Original Research Article

Contribution of Nature-Based Solutions to the Socio-Economic Resilience of Market Gardening in a Coastal Environment: the Case of the Saint-Louis Marine Protected Area (Senegal)

Abstract Aims: This study aims to analyze coastline mobility and evaluate the effects of soft solutions, such as typhavelles and reforestation, on market gardening in the Langue de Barbarie region, Saint-Louis, Senegal, an area highly vulnerable to coastal erosion, marine flooding, and soil salinization. Study Design: Observational and field-based study combining coastal geomorphological analysis and socio-economic surveys of local farmers. Place and Duration of Study: Langue de Barbarie, Saint-Louis, Senegal; fieldwork conducted between January and August 2022. Methodology: Eight (08) beach profiles were analyzed to assess coastline changes. Additionally, socio-economic surveys were conducted among market gardeners from affected villages to evaluate the impact of soft solutions on agricultural productivity. Field observations focused on the effectiveness of typhavelles in promoting sediment deposition and dune formation, while surveys assessed changes in marine flooding, soil salinization, crop yields, farmer incomes, and the number of active market gardeners. Results: Typhavelles enhanced sedimentation, increasing beach width and forming a dune cordon approximately 60 cm in average height through trapping of wind-blown sand. Surveys revealed a reduction in marine flooding and groundwater salinization. Consequently, agricultural yields improved, incomes increased, and the number of active market gardeners rose. These results demonstrate a clear link between the implementation of soft solutions and socio-economic resilience of coastal farming communities. Conclusion: Soft solutions, such as typhavelles and reforestation, are effective tools for mitigating coastal erosion while enhancing the socio-economic resilience of market gardening in coastal areas. These interventions represent a practical approach to adapting to climate change and protecting vulnerable littoral communities. Keywords: soft solutions, nature-based solutions, typhavelles, coastal erosion, beach profile, marine flooding, soil salinization.

1. INTRODUCTION

Coastal erosion is one of the major natural hazards affecting coastlines worldwide. It results from the interaction between natural factors—such as wave action, marine currents, winds, and sea-level variations—and anthropogenic factors, including coastal developments, sediment extraction, the construction of hydraulic structures, and the increasing urbanization of coastal areas (Bird, 1985; Paskoff, 2001). At the global scale, nearly 70% of sandy beaches are currently experiencing erosion, compared to only 10% undergoing accretion, while the remaining 20% exhibit relative stability (Bird, 1985).

Located at the interface between marine and terrestrial environments, coastal zones have long represented strategic spaces for human societies due to the diversity of resources and ecosystem services they provide, particularly in fisheries, agriculture, mineral extraction, energy production, and recreation (Gamblin, 1998). This attractiveness explains the high concentration of populations and economic activities in coastal areas worldwide. In West Africa, the coastline stretching from Mauritania to Gabon hosts approximately 105 million inhabitants, with nearly half of the urban population concentrated on only 5% of the surface area of coastal countries, generating about 56% of the regional gross domestic product (Croitoru et al., 2019; Brüning, 2022).

However, these coastal environments are now facing increasing environmental pressures, particularly related to climate change, pollution, overexploitation of natural resources, and coastal erosion. Although coastal erosion is a natural phenomenon driven by sedimentary dynamics, it is significantly amplified by anthropogenic activities such as the construction of dams that trap sediments upstream, sand mining for construction purposes, and rapid coastal urbanization. These combined factors accelerate shoreline retreat and increase the vulnerability of ecosystems and human activities.

Senegal, a coastal country in West Africa with a 718 km-long coastline, is not exempt from this issue. More than 52% of its population, approximately 7.8 million people, live in coastal zones (Sarraf et al., 2021). At the national scale, nearly 65% of the Senegalese coastline is affected by erosion, with an average shoreline retreat rate estimated at 1.6 m per year between 1984 and 2016 (Luijendijk et al., 2018, cited by Croitoru et al., 2019). However, the intensity and manifestations of erosion vary according to geomorphological contexts and local dynamics.

Among the most vulnerable areas is the city of Saint-Louis, often referred to as an “amphibious city” due to its location between the mainland, a fluvial island, and the Langue de Barbarie (Brüning, 2022). The opening of the breach on the Langue de Barbarie in 2003, initially intended to reduce fluvial flood risks, profoundly altered the hydro-sedimentary dynamics of the area (Sy, 2006). According to Diatta (2004), this breach—whose width expanded from a few meters to several kilometers—led to intensified coastal erosion, recurrent marine flooding, and increased salinization of soils and groundwater, resulting in the degradation of natural habitats and the disappearance of numerous market gardening farms (Durand et al., 2010; Dumas et al., 2006).

Paradoxically, the southward progression of the coastal spit has also led to the formation of new sandy lands characterized by the presence of a subsurface freshwater lens, thus offering favorable conditions for the development of market gardening (Sy, 2012; Sambou, 2017). These areas attract new agricultural populations but remain extremely fragile and exposed to coastal hazards, particularly sand encroachment, marine flooding, and salinization (Durand et al., 2010; Sambou, 2020).

In response to the intensification of coastal hazards, the Saint-Louis Marine Protected Area, in partnership with various institutional and technical stakeholders, has initiated the implementation of sustainable ecological solutions, based on the installation of typhavelles and reforestation activities, aimed at strengthening dune protection and soil stability. These measures primarily involve the installation of Typha palisades (“typhavelles”) over more than 1.5 km to promote sand trapping and the formation of a protective dune ridge, as well as reforestation actions designed to ensure long-term soil stabilization (Dumas and Mietton, 2006).

It is within this context that the present study is conducted, aiming to analyze the impacts of sustainable ecological solutions on the socio-economic activities of market gardening within the Saint-Louis Marine Protected Area in Senegal. The general objective is to assess the effects of these solutions on the morphodynamic evolution of the coastline and on the resilience of market gardening activities. Specifically, the study seeks to: (i) analyze the evolution of beach profiles monitored since the installation of typhavelles, and (ii) assess market gardeners’ perceptions of the effects of the implemented soft solutions.

2. material and methods

2.1. Study Area

The Saint-Louis Marine Protected Area (MPA) is located along the Langue de Barbarie and covers an area of approximately 496 km², extending between the municipalities of Saint-Louis and Ndiébène Gandiol (Sarr, 2020). The area includes a densely populated urban zone as well as several rural communities. The MPA is designed to conserve coastal and marine ecosystems, restore degraded habitats, and support local socio-economic activities, particularly fisheries and market gardening (Sarr, 2016).

Figure 1: Location of the study area

Fieldwork involved the use of positioning and measurement equipment (GPS, measuring tape), visual documentation tools (camera), land and maritime means of transport (vehicle and AMP patrol boat), as well as data collection and recording tools (survey forms administered to market gardeners, pencils, pens, and notebooks).

2.2. Literature Review

A comprehensive literature review was conducted to gain a better understanding of the study area and the research theme. This step made it possible to define the key concepts, formulate the research problem, and review soft (nature-based) coastal protection solutions. The review took into account several factors, including available time, logistical resources, and access to the field. The sources consulted included the AMP/SL library, the ISFAR library, as well as relevant online resources.

2.3. Beach Profile Data Collection

Data related to beach profiles were extracted from the monitoring database initiated in June 2019 for Profiles 1 to 4 and in September 2019 for Profiles 5 to 8. These data were processed using Excel software and collected on a monthly basis to analyze the evolution of the profiles between May 2019 and February 2023.

Location of the profiles is as follows:

Profile 1: Typhavelle prototype

Profile 2: 6th palisade facing the sea

Profile 3: 29th palisade facing the sea

Profile 4: Last palisade on the northern side

Profile 5: Central part of the 3rd compartment, near the April 2019 installations

Profile 6: Central part of the 3rd compartment, extreme northern section

Profile 7: Central part of the 3rd compartment, adjacent to the May 2019 installations

Profile 8: Central part of the 3rd compartment, extreme southern section

Figure 2: Location of the beach profiles

2.4. Socio-economic Data Collection

Socio-economic information related to market gardening activities was collected through semi-structured interviews with market gardeners from four villages surrounding the AMP/SL: Diéle Mbame, Keur Barka, Mambara, and Doun Baba Diéye. These villages were selected because the majority of market gardeners operating in the northern part of the Langue de Barbarie originate from these localities, as the AMP area itself is uninhabited.

2.5. Sampling Method

Given the limited number of market gardeners in these villages, a complete census survey was conducted among the 152 market gardeners identified by the AMP in 2021 (Table 1).

Table 1: Distribution of surveyed respondents by village

| Village | Diéle Mbam | Doun B. Diéye | Mambara | Keur Barka | Total |
| --- | --- | --- | --- | --- | --- |
| Market gardening | 83 | 50 | 2 | 17 | 152 |

The data collected were both quantitative (yields, unit prices, number of producers) and qualitative (descriptive observations derived from interviews).

Data Processing and Analysis Tools

- Microsoft Excel: used for data entry, processing, and organization of quantitative data, as well as for generating graphs and statistical tables.

- QGIS and Google Earth: used for mapping and spatial analysis of beach profiles and the areas where soft (nature-based) solutions were implemented.

- Sphinx: used for processing and analyzing socio-economic survey data, enabling the extraction of both quantitative and qualitative information and the generation of descriptive and cross-tabulated statistics.

3. results AND DISCUSSION

3.1. RESULTS

3.1.1. Effects of soft solutions on the coastline
Figure 3 illustrates the effects of soft solutions on the coastline. It shows:

- an initial decreasing trend in beach length, followed by an increase up to 117.5 m in August 2022;

- then a sharp drop to 3 m in October 2022, before a new phase of growth;

- finally, dune length (Ldune) reaches a peak of 127.5 m in October 2022, then decreases immediately thereafter.

Figure 3: Profiles of the effects of soft solutions on the coastline

Physical and morphodynamic explanation

Temporary sediment transfer phenomenon: The dune peak observed in October 2022 suggests a massive transfer of sediments from the beach (foreshore) to the dune. This can occur following episodes of intense swell or storms, during which waves erode the beach and redeposit sediments at the base or crest of the dune. This process results in a sharp reduction in beach width, while the dune gains in volume and length.

Role of meteorological conditions (swell, winds, tides): October corresponds, in many coastal areas, to the transitional period between the calm season and the storm season. Episodes of strong swell combined with high tides often cause frontal beach erosion. The sand removed from the beach is transported and temporarily accumulated higher up on the berm or on the dune, explaining the peak in dune length (Ldune).

Influence of “soft solutions” (such as typha–velles, vegetation, biodegradable stakes): These techniques aim to promote natural sand stabilization and encourage the formation of new embryonic dunes. During storm surges, they can slow marine encroachment and trap sand, resulting in a rapid but temporary increase in Ldune. After stabilization, sediments may be redistributed, leading to the subsequent decrease observed.

Cancellation of elevation in front of and behind the typhavelle: The fact that elevation was cancelled out in July 2021 indicates that, from that period onward, the structure was submerged or levelled by sediment inputs or withdrawals. This reflects a highly morphodynamically active zone, where soft solutions likely modified the local dynamics of sediment deposition and erosion.

3.1.2. Different crops cultivated by the respondents

The results obtained (Table 1) show that the main crop is cabbage (100%). However, 18% of the respondents also grow tomato, 15% watermelon, 13% sweet potato and hibiscus (bissap), and 8% okra

Table 1: Distribution of crops cultivated by the respondents

| Crops cultivated | Cabbage | Tomato | Watermelon | Sweet potato | Hibiscus | Okra |
| --- | --- | --- | --- | --- | --- | --- |
| Percentage of cultivation | 100% | 18% | 15% | 13% | 13% | 8% |

3.1.3. Effects of soft solutions on crop yields
According to Table 2, before the installation of soft solutions, 69.5% of market gardeners had yields below 1 t, 29.3% had yields between 1 t and 4 t, and only 1.2% exceeded 4 t. After the implementation of these solutions, yields increased considerably: 64.4% of market gardeners now produce between 1 t and 4 t, 25.4% produce more than 4 t, while those with yields below 1 t now account for only 10.2%.

Table 3: Yield classes before and after the implementation of soft solutions.

| Yield (t) | Less than 1t | 1t to 4t | More than 4t | Total |
| --- | --- | --- | --- | --- |
| Before | 69.5% | 29.3% | 1.2% | 100% |
| After | 10.2% | 64.4% | 25.4% | 100% |

3.1.4. Effect of soft solutions on market gardeners’ income

According to Table 3, before the installation of soft solutions, 59.3% of market gardeners had an income between 100,000 and 300,000 CFA francs, 25.4% earned less than 100,000 CFA francs, 12.3% earned between 300,000 and 500,000 CFA francs, and only 3% exceeded 500,000 CFA francs. After the implementation of soft solutions and the increase in yields, incomes also improved: 56.1% of market gardeners now earn more than 500,000 CFA francs, 24.3% earn between 300,000 and 500,000 CFA francs, while the proportion of producers earning between 100,000 and 300,000 CFA francs and less than 100,000 CFA francs declined to 16% and 3.6%, respectively.

Table 4: Income classes before and after the implementation of soft solutions.

| Income (CFA francs) | More than 100,000 | 100,000 to 300,000 | 300,000 to 500,000 | Than 500,000 | Total |
| --- | --- | --- | --- | --- | --- |
| Before | 25.4% | 59.3% | 12.3% | 3% | 100% |
| After | 3.6% | 16% | 24.3% | 56.1% | 100% |

The effect of soft solutions on the number of vegetable farmers

Among the 152 surveyed vegetable farmers, all reported fewer than 100 farmers before the implementation of soft solutions, with fewer than 60 farmers present on the site. Since the introduction of these measures, the number of farmers has steadily increased. Currently, 83% of respondents report between 100 and 200 farmers, while 17% indicate that the number exceeds 200.

3.2. DISCUSSION

Based on the analysis of the beach profiles, we observe that the overall trend in beach length is indeed a reality, except for profile 6 where the length has decreased compared to its initial state. It should be noted that the beach length at the beginning of the construction was significantly shorter than the current length. We also note a retreat of the beach along almost the entire length of the breakwater on February 15, 2021, which was rapidly restored the following month. The decrease in beach length observed at this time (February 15) and at profile 6 is due to coastal erosion (Koukpémédji, 2022). With the force of swells and waves, the sea erodes the beach, causing the shoreline to retreat inland due to the loss of material. According to Faye (2022), coastal erosion is a large-scale phenomenon affecting all countries, but particularly sandy coasts, as is the case on the Langue de Barbarie spit. The period between 2009 and 2019 was marked by significant coastal erosion at a very high rate, with an average annual retreat rate of 5.48 m/year. The increase in beach length along the entire structure is due to sedimentation linked to the deposition of windblown sediments in the spits, originating from coastal drift as well as from Mauritania ( Faye, 2022). This system has thus enabled the reconstitution of the dune system along the beach, reducing marine flooding in the market garden plots. It is within this same framework that the beach length has increased over the years. The structure has therefore effectively fulfilled its role in trapping windblown sand. These results are similar to those of Musereau et al. (2008), who described the installation of sand fences (similar in structure to typhavelles) at the Saint-Trojan dune in France. The installation consisted of installing sand fences in the form of a wind-permeable curtain with a permeability ratio of approximately 50%. This reduced wind speed and allowed for sand deposition in front of and behind the structure. This intervention contributed to beach accretion and its seaward advance. From north to south of the dune massif, at its maximum extent, the advance into the sea varies from 500 to 1,700 m. These results allow us to conclude that the typhavelles helped combat coastal erosion by promoting beach widening and dune formation through the trapping of windblown sand within the traps.

The effects of the typhavelles (a type of coastal erosion control measure) are not limited to the beach area; combined with the reforestation of casuarina trees, market gardening has been able to develop behind them. In the four villages concerned—Diéle Mbam, Doun Baba Diéy, Keur Barka, and Mambara—all the market gardeners are familiar with the sustainable solutions and have clearly seen their benefits. According to the distribution of the market gardeners, the majority are between 40 and 60 years old. This shows that adults and the elderly are more involved in market gardening. Rural exodus and migration mean that young people are practically inactive in market gardening. According to Brüning (2022), to cope with the environmental degradation caused by the construction and widening of the breach, many young people have chosen migration as an adaptation strategy. More than half of the producers are men (54%), but the percentage of women (46%) shows that they are also involved in this activity and, according to them, have their own plots within the Marine Protected Area (MPA).

According to the survey results, cabbage is the main crop grown and sold in this area. Other crops such as tomatoes, hibiscus, and okra are generally grown for consumption. Before the implementation of sustainable solutions, the market garden plots were submerged by seawater during periods of heavy swells and strong winds, leading to the salinization of freshwater aquifers. As a result, more than half of the producers (69.5%) had a yield of less than 1 ton, and 59.3% had an income between 100,000 and 300,000 FCFA. The work of Niang et al. (2020) shows that low yields and plot abandonment in the Gandiolais region are due to soil salinity. However, after the implementation of these solutions, 64.4% of market gardeners have yields between 1 and 4 tons, 25.4% have yields exceeding 4 tons, and more than half (56.1%) have an income exceeding 500,000 CFA francs. There has also been an increase in the number of producers; according to respondents, the majority (83%) reported between 100 and 200 market gardeners, while 17% reported more than 200. This demonstrates that the solutions have a positive impact on market gardening

4. Conclusion

This study, focusing on the effects of soft engineering solutions on the socio-economic activity of market gardening, was conducted in the Saint-Louis Marine Protected Area (MPA) and in the villages of Diéle Mbam, Doun Baba Diéy, Keur Barka, and Mambara. In accordance with the specific objectives, the results indicate that a positive evolution in beach length was observed across all profiles, primarily due to sedimentation. Profile 6, however, is an exception, showing a regression attributable to coastal erosion. Furthermore, the survey results reveal that soft engineering solutions have a beneficial effect on market gardening, resulting in a significant increase in yield, income, and the number of producers.

Consent

All authors declare that this study does not involve human patients and, therefore, no specific informed consent is required for the publication of this article.

Ethical approval

This study does not involve human patients or animal subjects. Therefore, the ethical approvals required for medical or animal research, as described above, are not applicable to this manuscript.

References

- Benkhaldoun, Z. (2019). Experimental studies of cross-shore profiles of sandy beaches [Unpublished manuscript]. Academia.edu. https://www.academia.edu/47868494/Etudes_exp_rimentales_des_profils_cross-shore_de_plages_sableuses

- Brüning, L. (2022). Coastal erosion in northern Senegal: Migration and adaptation strategies (Doctoral dissertation). Institute of Geography, University of Neuchâtel, Switzerland.

- Croitoru, L., Miranda, J. J., & Sarraf, M. (2019). The cost of coastal zone degradation in West Africa. The World Bank.

- Diatta, I. (2004). The opening of a breach across the Langue de Barbarie (Saint-Louis, Senegal): Public authorities and consequences of the rupture (Master’s thesis). Gaston Berger University, Saint-Louis, Senegal.

- Dumas, D., & Mietton, M. (2006). Hydrological modification and flood risk reduction in Saint-Louis following the opening of the breach. Revue de Géographie Alpine / Journal of Alpine Research.

- Durand, P., Anselme, B., & Thomas, Y.-F. (2010). The impact of the 2003 breach opening in the Langue de Barbarie at Saint-Louis, Senegal: A change in the nature of flood hazard? Cybergeo: European Journal of Geography, 496. https://doi.org/10.4000/cybergeo.23017

- Faye, I. B. N. (2010). Shoreline dynamics on the sandy coasts from Mauritania to Guinea-Bissau (West Africa): Regional and local approaches using photo-interpretation, image processing, and historical map analysis (Doctoral dissertation). University of Western Brittany, Brest.

- Faye, M. (2022). Socio-economic impacts of coastal erosion on the Langue de Barbarie: Goxxu Mbacc, Santhiaba, Guet Ndar [Academic thesis].

- Gamblin, A. (1998). The coasts – living spaces. Sedes.

- Hervy, S. (2012). Automatic beach profile analysis tool. HAL. https://hal.science/hal-03857017/file/17_40_hervy.pdf

- Koukpémédji, E. S., Laïbi, R. A., & Vissin, E. W. (2022). Morphological dynamics of the beach protected by the submerged breakwater at Avlékété (South Benin). European Scientific Journal, 18(16), 109. https://doi.org/10.19044/esj.2022.v18n16p109

- Luijendijk, A., Hagenaars, G., Ranasinghe, R., Baart, F., Donchyts, G., & Aarninkhof, S. (2018). The state of the world’s beaches. Scientific Reports, 8, 6641. https://doi.org/10.1038/s41598-018-24630-6

- Musereau, J., & Regnauld, H. (2008). On the behavior and management of inherited coastal forms from strong nature/society interaction (Charente estuary area, France). Géomorphologie: Relief, Processus, Environnement, 14(4), 245–256.

- Niang, S., Sy, A. A., Faye, C. A. T., Sow, S. A., & Sy, B. A. (2020). Hydrogeochemical analysis of the spatial distribution of water salinity in the water table and Quaternary sands of the Niayes sector in Gandiolais (Northern Senegalese coast). European Scientific Journal, 16(24).

- Paskoff, R. (2001). Sea level rise and coastal spaces: Myth and reality. Oceanographic Institute, “Propos” collection.

- Sarr, M. (2022). The Saint-Louis Marine Protected Area – Overview and conservation challenges. Paloc.

- Sarr, S. M., et al. (2016). Restoration of degraded ecosystems in the Saint-Louis Marine Protected Area through artificial reef deployment and mangrove reforestation. Journal of Applied Sciences.

- Sy, B. A. (2005). The opening of the Langue de Barbarie breach and its consequences: A geomorphological approach. Annals of the Faculty of Letters, Languages, Arts, and Human Sciences (FLASH), University of Bamako.

- Sy, B. A. (2012). Evolution of the breach on the Langue de Barbarie and its consequences [Report/Lecture]. Gaston Berger University of Saint-Louis, Senegal.

- Sy, B. A., Sy, A. A., Bodian, A., Faye, A., Niang, T., Diop, S., & Ndiaye, M. (2015). Open breach on the Langue de Barbarie in Saint-Louis: Outline of an emergency management assessment. Physio-Géo, 9, 197–216.