Original Research Article

STATUS OF WATER QUALITY OF NAIKAL POND IN SAGARANADU, NORTHEASTERN KARNATAKA, INDIA

Abstract

The study was conducted to determine the quality of Naikal pond water in northeastern Karnataka. Water sample was collected near deepest part of pond. The collected water sample was subjected to characterization of physico-chemical parameters. viz., temperature, pH, electrical conductivity, total dissolved solids, total alkalinity,dissolved oxygen, biological oxygen demand, total hardness, calcium, magnesium, chloride, sulphate, nitrate and phosphorous. The results were evaluated and compared with BIS drinking water quality standards. The most of the water quality parameters were within the permissible level and water is suitable for domestic purpose. From the data obtained, the water quality index was calculated to quantify overall pond water quality status of the area. It is found that based on the WQI results the samples are falling under excellent category and hence suitable for domestic purposes.

Keywords: Physico-chemical parameters, Pond water, Water quality, Drinking water standards

1. Introduction

Freshwater ecosystems including with ponds and lakes exhibit complex physico-chemical dynamics that play a crucial role in determining the overall health and ecological balance of  ecosystems(Lakhwinder Singh et al., 2025). A pond is referred to as a man-made or natural water body that holds water for few months  or round the year (Mushtaq et al., 2006; Passy et al., 2012). These ponds are one of the surface water bodies which are the important resources of fresh water. Since village ponds used to be a lifeline for people in rural areas and utilized for agriculture, aquaculture, bathing and drinking(Hosamani & Mahantesh, 2024). The scarcity in the surface water resources makes the people dependent on the underground water for the regular water supply. In this background, ponds are one of the most important water resources for human consumption. Pond ecosystems have thephysico-chemical parameters which play a vital role in determining the distribution pattern and quantitative abundance of organisms(Rasmita Padhy et al., 2019; Rachna Bhateria &  Disha Jain, 2016). The water quality in the water bodies is greatly dependent on the physical, chemical and biological parameters and their interaction with each other. The enrichment of nutrient from natural aging or by the influx and sedimentation of organic components from human activities and agricultural runoff leads to deterioration of health of ecosystems(Basavaraj & Kadadevaru, 2024). On the other hand, these ponds can reduce flood impacts during rainy seasons and facilitate groundwater recharge, enhancing the availability of water throughout the year nutrient retention, carbon sequestration, fish production, eliminates diffuse pollutants including phosphous and nitrogen, and habitat conservation(Williams et al. 2020; Oertli and Parris 2019; Cereghino et al. 2014; Moore and Hunt, 2012). The ecological value of ponds is often underestimated. On the other hand, healthy pond ecosystems are critical for achieving several sustainable development goals (SDG 6, 13, 14). Therefore, the present study aims to assess the physico-chemical parameters of Naikal pond of Wadagera for its suitability for various purpose.

2. Materials and Methods

2.1 Study Area

Naikal pond (16º12´50.00" N, 77º03'29.82" E) is located in Wadagera taluk, Yadgir district of northeastern Karnataka in peninsular India (Fig. 1). The water body is situated 15 km away from the Wadagera taluk head quarter. It is a man-made perennial lake. The water is used for irrigation and fishing and it is an habitat for many aquatic and avian fauna.The normal annual rainfall is around 636 mm. The average ambient temperature fluctuated between 16.7ºC to 40ºC.

Fig. 1 Satellite view of Naikal pond

2.2 Collection of water sample

Water sample was collected using pre-sterilized polyethylene bottles from Naikal pond especially at morning hours during pre-monsoon season for a period of two years(March  2022 & 2023. Collected sample was transferred in an ice box to the laboratory for further analysis of various physico-chemical parameters of water sample. Air and water temperature, pH and dissolved oxygen were recorded on spot using Water analyzer kit(Systronis). Water collected in BOD bottle was shifted to laboratory for incubation.

2.3 Analysis of physico-chemical parameters

It is very essential and prerequisite to test the water characteristics before it is used for various purposes like domestic, irrigation and industry. Water must be tested for its different physico-chemical parameters. Water must be free from solids(floating, dissolved and suspended) and microbial as well as bacteriological impurities. Hence, water has been tested for the following  characteristics viz, temperature, pH, turbidity, electrical conductivity, total dissolved solids, alkalinity, dissolved oxygen, biological oxygen demand, total hardness, calcium, magnesium, chloride, sulphate, nitrate  and phospahate as per the standard protocol(APHA, 2012, BIS, 1991).

2.4 Calculation of the Water quality index (WQI):

Water quality index (WQI) is one of the most used tools to describe water quality. It is based on physical, chemical, and biological characteristics that are combined into a single value that ranges from 0 to 100(Sandra Chidiac et al., 2023). It is used for assessment of the quality of water various water resources such as rivers, streams and lakes, etc (Chatterjee and Raziuddin, 2002; Singh et al., 2015). Bureau of Indian Standards for drinking water quality have been used to calculate the water quality index .

The three steps of water quality index are as follows;

Step 1

wi= according to relative importance (overall quality of water for drinking purpose). The maximum weightage of 5 has been assigned to nitrate parameter and minimum weightage of 2 has been assigned for calcium and magnesium (may not be that harmful)

Step 2

Relative weight(Wi)

Si=Wixqi

qi=CiSi=100

Where qi = Quality rating, Ci = concentration of each chemical parameter of sample in mg/L, Si = Bureau of Indian standards(drinking) for each chemical parameter in mg/L. Water quality index (WQI) = sub index of the Ith parameter

Step 3

SIi=Wixqi

Where SIi = the SI is first determined for each chemical parameter which is then used to determine the WQI as per the following equation

WQI=SIi

Table 1. Parameters, weight factors, and limit values considered for the water quality index (Ramakrishnaiah et al., 2009)

| Parameter | Units | Weight (wi) | Relative weight (Wi) | Limits value |
| --- | --- | --- | --- | --- |
| pH | - | 4 | 0.13333 | 8.5 |
| TDS | mg/L | 4 | 0.13333 | 2000 |
| Alkalinity | mg/L | 3 | 0.10000 | 600 |
| Hardness | mg/L | 3 | 0.10000 | 600 |
| Ca2+ | mg/L | 2 | 0.06667 | 200 |
| Mg2+ | mg/L | 2 | 0.06667 | 100 |
| Cl- | mg/L | 3 | 0.10000 | 1000 |
| SO42- | mg/L | 4 | 0.13333 | 400 |
| NO3- | mg/L | 5 | 0.16667 | 100 |
|  |  | wi=30 | Wi=1 |  |

3. Results and Discussion

The concentrations of various physico-chemical parameters of Naikal pond water was depicted in Table 1. In India the temperature is relatively high during the pre-monsoon season but with the arrival of the southwest monsoon water temperature reaches its lowest value (26°C) in August (Dwivedi et al., 1974). Aquatic organisms are dependent on certain temperature ranges for their optimal health. Temperature affects the oxygen content of the water, the rate of photosynthesis, the metabolic rates of aquatic organisms (Rachna Bhateria and  Disha Jain, 2016). In the present study, air temperature was recorded from 32ºC to 34ºC whereas, water temperature was recorded from 26ºC to 27ºC. In the present study, the recorded average values of temperature indicates that the water quality would not be affected by this parameter.

The pH is measured mathematically by, the negative logarithm of hydrogen ions concentration. pH plays a role in the growth of flora and fauna of aquatic ecosystems(Hosamani & Mahantesh, 2024). pH value of pond was ranged between 7.51 (March 2022) to 7.67 (March 2023) during the study period. In comparison two years, pH did not change much. The electrical conductivity of water is a direct function of its total dissolved salts in water. EC fluctuated between  818 µS/cm and 1020 µS/cm with an average of 919 µS/cm. Total alkalinity is the buffering capacity of water, it is mainly due to carbonate and bicarbonates of calcium, magnesium, potassium and sodium and other bases. The alkalinity ranged from 278 mg/L to 394 mg/L with an average value of  336 mg/L. Total alkalinity value considerably  exceeded the BIS standards(200 mg/L) in March 2023.

Dissolved oxygen (DO) are a very important and crucial chemical parameter that affects the survival of aquatic life. Additionally, low DO levels could have negative impact on other aquatic organisms such as insects and crustaceans, larval forms etc,. DO levels ranged between 4.05 mg/L and 4.26 mg/L during  1 & 2 year. More than 4.0 mg/L is DO supports fish populations in aquatic water bodies. Biological oxygen demand is a measure of amount of oxygen required by the group of microorganisms to decompose the organic matters present in a water body(Nusrat and Shahnaz, 2020). BOD was ranged between 2.03 mg/L and 2.11 mg/L in both the years. Similarly, Basavaraja and Kiran recorded an average of 2.80 mg/L BOD recorded in Sagara lake during pre-monsoon season. According to Ekubo and Abowei (2011) aquatic system with BOD levels between 1.6 and 2.0 mg/L considered clean.

Total hardness level was 188 mg/L and 222 mg/L in two years and had a mean of 201 mg/L. It doesn't exceeded the BSI standards (300 mg/L) in both the years. According to Sawyer and McCarthy(1967), water having total hardness between 150 and 300 mg/L are considered as hard water. The water in this study is hard water. Similar observation was made by Basavaraja et al.,(2017) in Shettikera lake during pre-monsoon season. The growth and dynamics of populations of fresh water flora and fauna gets affected due to both calcium and magnesium (Vikas Jain and Sunil Kumar, 2021). The values recorded for calcium and magnesium  in present study ranged from 37.02 to  41.64 mg/L and  from 22.03 to 27.52 mg/L  respectively. All values of both parameter were within the permissible level.

An excessive level of chloride can damage aquatic organisms by hampering  their osmoregulation process, which allows organisms to regulate the salts level in their body fluids (Hunt et al. 2012). Chloride value ranged between 98.01 mg/L and 117.72 mg/L and it is well within the permissible level. The sulphate concentration of the water samples varied from 82.56 to 84.48 mg/L. The result reveals that the pond water has permissible range of sulphate ions. The presence of nitrogen content in excess is an indicator of organic pollution. It is originated from the added nitrogenous fertilizers, and decay of dead plants and animals. Further, they are all oxidized to nitrate form by natural process(Balaji Prashanth et al., 2013). Nitrate levels are normally stabilized in the 50-100 ppm range(Debalakshmi Das, 2019). In the present study, nitrate content was recorded from 0.461 to 0.488 mg/L during March 2022 & 2023. However it remains static in last year's where variation was not so significant. Low concentrations of phosphate(0.003 mg/L & 0.006 mg/L) was recorded. As per the water quality index  belongs good water quality in both the years (Table 2).

Table 2. Physico-chemical parameters of Naikal pond water

| Parameter | March 2022 | March 2023 | Average | BIS Standards |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Permissible level | Excessive level |
| Air temperature (ºC) | 32 | 34 | 33 | - | - |
| Water temperature (ºC) | 26 | 27 | 26.5 | - | - |
| pH | 7.51 | 7.67 | 7.59 | 6.5-8.5 | 6.5-9.2 |
| Electrical conductivity(µS/cm) | 818 | 1020 | 919 | - | - |
| Total dissolved solids (mg/L) | 373 | 470 | 421.5 | 500 | 1500 |
| Alkalinity (mg/L) | 278 | 394 | 336 | 200 | 600 |
| Dissolved oxygen (mg/L) | 4.05 | 4.26 | 4.155 | - |  |
| Biochemical oxygen demand (mg/L) | 2.03 | 2.11 | 2.07 | - |  |
| Total Hardness (mg/L) | 180 | 222 | 201 | 300 | 600 |
| Calcium (mg/L) | 37.02 | 41.64 | 39.33 | 75 | 200 |
| Magnesium (mg/L) | 22.03 | 27.52 | 24.775 | 30 | 100 |
| Chloride (mg/L) | 98.01 | 117.72 | 107.865 | 250 | 1000 |
| Sulphate (mg/L) | 82.56 | 84.48 | 83.52 | 200 | 400 |
| Nitrate (mg/L) | 0.461 | 0.488 | 0.4745 | 0-60 | 60 |
| Phosphate (mg/L) | 0.003 | 0.006 | 0.0045 | - | - |

Table 3: Water Quality Index and Status of Water Quality (Chatterji and Raziuddin 2002)

| Water Quality | Water Quality Status | WQI Values |  |  |
| --- | --- | --- | --- | --- |
|  |  | March 2022 | March 2023 | Average |
| 0 – 25 | Excellent Water Quality | - | - | - |
| 26-50 | Good Water Quality | 28.41 | 32.73 | 30.57 |
| 51-75 | Poor Water Quality | - | - | - |
| 76-100 | Very Poor Water Quality | - | - | - |
| > 100 | Unfit for drinking | - | - | - |

4. Conclusion

Observed values of pond water quality parameters viz.  pH, total dissolved solids, total hardness, calcium, magnesium, chloride, sulphate and nitrate levels were well within the permissible level except alkalinity, however, its level is within the excessive level. Dissolved Oxygen level was above 4 mg/L during summer which supports heterotrophic life of pond ecosystem. The water quality index confirmed this pond water belonging to good water quality category. This study help concerned authority to protect and conservation of small water bodies because, quality of water is essential for both the flora and fauna and maintenance of equilibrium status. .

Ethical Approval

The author(s) have declared ethics committee/IRB approval is not relevant to this content.

Consent- Not applicable

References

- APHA. (2012). Standard Methods for the Examination of Water and Wastewater. 22nd Edition, American Public Health Association, American Water Works Association, Water Environment Federation, Washington, D.C..

- B. Balaji Prashanth, R. Nadakumar, S. Dinesh Kumar, S. Ananth, A. Shenbagha Devi, T. Jayalakshmi, P. Raju, M. Thiyagarajan, and P. Santhanam. 2013. Seasonal variations in physico-chemical characteristics of pond and ground water of Tiruchirapalli, India, Journal of Environmental Biology, 34, 529-537.

- Basavaraj, S.K., & Girish G. Kadadevaru.(2024).  Evaluation of water quality Parameters and zooplankton community during pre- and post-Covid phases at Chandravalli Lake, Chitradurga District, Karnataka, India. International Journal of Ecology and Environmental Sciences, 50, 83-96.

- Basavaraja D., & B. R. Kiran. (2016). Seasonal variations in physico-chemical parameters of Sagara Lake, Yadgir District, Karnataka. International Journal of Applied and Pure Science and Agriculture (IJAPSA), 02(09), 79-83.

- Basavaraja, D., Doddabasawa, K., & M. Sharanagouda.(2017). Water quality status and its suitability for drinking purpose in Shettikera Lake, Yadgir District, Karnataka. Environment & Ecology, 35 (4B), 3046-3050.

- Bureau of Indian Standards. (1991).Specification for drinking water. IS: 10500, New Delhi, India..

- Cereghino, R., Boix, D., Cauchie, H.M,, Martens, K., & Oertli, B.(2014). The ecological role of ponds in a changing world. Hydrobiologia, 723, 1-6. doi: 10.1007/s10750-013-1719-y.

- Chatterjee, C., and Raziuddin, M. (2002). Determination of water quality index (WQI) of a degraded river in Asansol Industrial area, Raniganj, Burdwan, West Bengal. Nature, Environment and Pollution Technology,1 (2), 49-59.

- Chen, W., He B, Nover, D., et al. (2019) Farm ponds in southern China: Challenges and solutions for conserving a neglected wetland ecosystem. Science of the Total Environment, 659, 1322-1334. doi: 10.1016/j.scitotenv.2018.12.394.

- Debalakshmi Das. (2019). A Study on Water Quality for Management of Pond Fish Culture. International Journal of Basic and Applied Biology,6(3), 235-245.

- Dwivedi, S.N,, Parulekar, A.H., Goswami, S.C., & Untawale, A.G. (1974). Ecology of mangrove swamps of the Mandovi estuary, Goa. In: G. Welsh, S. Snedaker and H. Teas (Eds.), Proc. Int. Symp. Biology & management mangroves. 1,115-1125.

- Ekubo,  A. A and Abowei J.F.N. (2011). Review of some water quality management principles in culture fisheries. Research Journal of Applied Sciences, Engineering and Technology,3(2), 1342-1357.

- Hunt, M., Herron, E., & Green L. (2012).Chlorides in Fresh Water. College of the Environment and Life Sciences, The University of Rhode Island Kingston.

- Indermuehle, N., Oertli, B., Biggs, J., et al., (2008). Pond conservation in Europe: the European Pond Conservation Network (EPCN). SIL Proceedings, 1922-2010, 30, 446–448. 10.1080/03680770.2008.11902163.

- Lakhwinder Singh., Ramandeep Achint., & Veerpal Kaur. (2006). Studies on Physico-Chemical Parameters of Water in Ponds and Lakes of India. International Journal for Multidisciplinary Research (IJFMR),  7(1), 1-7.

- Mushtaq, S., Dawe, D., Lin, H., & Moya, P. (2006).  An assessment of the role of ponds in the adoption of water-saving irrigation practices in the Zhanghe Irrigation System, China. Agricultural Water Management, 3(1-2), 100-110.   https://doi.org/10.1016/j.agwat.2005.10.004

- Nusrat Ara., & Shahnaz Jamil. (2020). Analysis of selected physico-chemical parameters of two ponds of madhubani. Indian J.Sci.Res., 10(2), 85-91.

- Oertli, B., & Parris, K.M. (2019). Review: toward management of urban ponds for freshwater biodiversity. Ecosphere. Appendix 2, 1-4. doi: 10.1002/ecs2.2810.

- Passy, P., Garnier, J., Billen, G., Fesneau, C., & Tournebize, J. (2012). Restoration of ponds in rural lands capes: modelling the effect on nitrate contamination of surface water (the Seine River Basin, France). Science of The Total Environment, 430, 280-290. https://doi.org/10.1016/j.scitotenv.2012.04.035.

- Rachna Bhateria., &  Disha Jain. (2016). Water quality assessment of lake water: a review. Sustain. Water Resour. Manag., 2, 161-173. DOI 10.1007/s40899-015-0014-7.

- Ramakrishnaiah, C.R., Sadashivaiah, C. and Ranganna, G. (2009). Assessment of Water Quality Index for the Groundwater in Tumkur Taluk, Karnataka State, India. E-Journal of Chemistry, 6(2), 523-530.

- Rashmi Hosamani., & Vasudha Mahantesh. (2024). Physicochemical and biological water quality assessment of local ponds of Tumakuru, Karnataka, India. J Adv Sci  Res., 15(10), 15-19. DOI:10.55218/JASR.2024151003.

- Rasmita Padhy, Manoranjan Routray, Rajesh Kumar Sahoo, P.Sabar, and B.K Mohanty. (2019). Physicochemical Analysis of Pond Water in Berhampur Town, Odisha. Res J. Chem. Environ. Sci., 7(5-6), 15-19.

- Sandra Chidiac, Paula El Najjar, Naim Ouaini, Youssef El Rayess, & Desiree El Azzi. (2023). A comprehensive review of water quality indices (WQIs): history, models, attempts and perspectives. Rev Environ Sci Biotechnol,11, 22(2), 349-395. doi: 10.1007/s11157-023-09650-7.

- Sawyer, C.N., & McCarthy D.L. (1967). Chemistry of Sanitary Engineers, 2nd, MC Graw Hill, New York, pp. 518.

- Singh, S., Raju, N.J., and Ramakrishna, C.H. (2015). Evaluation of Groundwater Quality and Its Suitability for Domestic and Irrigation Use in Parts of the Chandauli-Varanasi Region, Uttar Pradesh, India. Journal of Water Resource and Protection, 7, 572-587.

- Vikas Jain., & Sunil Kumar. (2021). Assessment of quality parameters of a pond water: A Limnological case study in Bareilly District. RA Journal of Applied Research,  07(08), 2477-2484.

- Williams, P., Biggs, J., Stoate, C., Szczur, C., Brown, C.D, Bonney, S. (2020). Nature based measures increase freshwater biodiversity in agricultural catchments. Biological Conservation. 244,108515. doi: 10.1016/j.biocon.2020.108515.