Minireview Article

Banana wastes: A sustainable and cost-effective clean-up tool for various environmental challenges- A MINI REVIEW

ABSTRACT

The banana, a fruit edible all over the world and native to Southeast Asia, is a member of the genus Musa. In a variety of sectors, bananas are used to make juices and as product additives.  Banana wastes consist of the fruit peels which are usually discarded, rejected or overripe fruits, pseudostems, leaves, inflorescences, and residues generated during processing and fiber extraction. The peel, a byproduct of bananas, is removed after the fruit has been put to use. It weighs between 35 and 38 percent of the fruit. If the peel is not used effectively, it may become an environmental nuisance. There are issues specific to the environment at different levels and in different sectors. Causing foul odors, attracting pests, promoting microbial proliferation, and contributing to soil and water pollution due to uncontrolled decomposition are some of the environmental nuisance that can be created when banana wastes are improperly disposed of. The use of banana wastes as an affordable solution for a range of environmental issues is covered in this review.

1.0 INTRODUCTION

While fruits can be found all over the world, some are only available in a few nations. On the other hand, bananas are among the most widely consumed fruits and are cultivated in many nations across the world. Banana is considered the eighth-most important fruit globally, from 135 countries, 148 million tons were produced; It is important for food security and used as a staple food for over 400 million people worldwide (Thangavelu et. al., 2021).

Plantains and sweet bananas are the two varieties of bananas that are members of the Musaceae family. Its species are divided into 50 groups and are diverse around the planet. Cultivated bananas are known by their scientific names, Musa accuminata, Musa paradisiaca, and Musa balbisiana. Averagely, it weighs 125 grams, of which 75% is thought to be water and 25% is dry matter (Hikal, 2022).

Banana is one of the most widely cultivated crops across tropical regions, including Nigeria, where large-scale plantations make it an abundant and cost-effective agricultural resource. Products such as banana chips and banana figs are commonly manufactured by numerous small- and medium-sized enterprises throughout the country (Hassan et. al., 2018). However, these processing activities generate significant quantities of waste, with the peel accounting for approximately 30–40 grams per 100 grams of fruit, which translates to around 200 tons of waste daily—a volume that is projected to increase over time (Hassan et. al., 2018).

Rattanapan and Ounsaneha, in their study observed some types of waste in banana production, including rejected banana material, wastewater, empty banana bunch, banana peel, and waste generated from the production process (Rattanapan and Ounsaneha, 2022). Due to increase in population and demand, banana waste was recorded to have increased with over 353,443 tons per year, during the past ten years (Rattanapan and Ounsaneha, 2021). When banana waste is allowed to biodegrade naturally, toxic gases are released into the environment while producing foul odour (Housagul et. al., 2014). Banana peels, which make up 35–38% of the fruit, are by-product (Wafaa et al., 2022), used as fertilizer, animal feed and are thrown away, which is a problem for the environment. Making effective use of banana waste can positively impact both the environment and the economy by reducing greenhouse gas emissions and the carbon footprint (Ganesh et. al., 2021).

To give plants the nutrients they need to flourish, both organic and inorganic fertilizers are added to the soil. Although organic fertilizers are eco-friendlier and efficient, the expensive cost limits its adoption by farmers. Due to this, banana peels can be used to make an inexpensive, natural fertilizer. Waste materials can either be biodegradable or non-biodegradable; those that naturally break down or decompose and are environmentally- friendly are known as biodegradable wastes while non-biodegradable damage the environment and also takes a long time to break down (Weshah and Al-Hafud, 2023). It has been reported that the use of organic wastes like banana wastes enhances bioremediation of crude oil polluted soil by facilitating soil aeration and raising the water holding ability of the soil (Hatzikiseyian, 2017). In a crude oil polluted environment, there is an imbalance of nitrogen (N) and phosphorus (P), which may retard the growth of microbes (Nwogu et al., 2015); it is therefore of prime importance that appropriate concentrations especially of the two nutrients are maintained in the optimal ratio in order to offset the imbalance caused by high carbon content of crude oil. Banana wastes which have been found to contain N and P (Tavali, 2022) would come handy here to offset this imbalance.

Synthetic fertilizers have been applied as biostimulants over time, for improved bioremediation of sites contaminated by petroleum hydrocarbons. However, unfavourable effects like eutrophication and air pollution were caused by its overuse. Banana peel waste is currently being investigated within the framework of a circular economy to encourage waste recycling and lessen environmental impact because of its various bioactive qualities and also being used to advance sustainable development objectives. Banana wastes are applied to relieve the environment in some ways, they are: banana fiber has recently been employed by the textile industry to promote the use of natural fiber in the creation of fabrics (Provin et. al., 2024); along with new banana leather, banana fiber is also utilized in the paper, cardboard, and packaging sectors. Paper made from banana fiber has been used to create wallets and artwork (Tewari, et al., 2024). Biodegradable pots, biofertilizers, food preservatives, functional foods, antioxidants, and antimicrobial medication components are just a few of the uses for banana peel waste in the food, pharmaceutical, and agricultural sectors (Manzoor, and Ahmad 2021; Afzal et. al., 2022; Shaukat et. al., 2023; Narwal et. al., 2024). Environmental impact was highly presented by Thailand as carbon footprints and water, with a high volume of banana waste production (Rattanapan and Ounsaneha, 2022).

2.0 BANANA WASTES

After banana production and usage, waste is generated in several categories; this includes wastewater, banana peel, empty banana bunch, the rejected banana material and waste from the production process (Rattanapan and Ounsaneha, 2022). The losses and waste from banana can reach up to 35% in the supply chain for banana production (Sagar et. al., 2018). In the last decade, an increase in banana waste was recorded, with over 353,443 tons annually (Rattanapan and Ounsaneha, 2022).

2.1 WASTEWATER:

According to Rattanapan and Ounsaneha (2022), the cleaning process of raw material with 511m3 of wastewater annually generates the waste of banana production; from this same study, banana raw material can be rejected as a result of inappropriate weight or unsuitable size and skin. From the selection process alone, about 131,400-350,400 kilograms of bananas were discarded. The study further deduced that during the production process, waste banana peel and empty banana bunch generated was 276,000-360,000 kilograms annually.

2.2 BANANA PEEL

The banana peel or outer covering that shields the banana fruit, is a fibrous protective layer known for its signature yellow colour that turns brown as it ripens. Banana peel is a by-product of the food processing industry and home consumption. Since it makes up around 40% of the fruit in weight, using or consuming bananas results in a significant annual generation of banana peel (Vu et.al., 2017). Throughout history, it has been utilized in animal feed, cooking, and medicine. Nonetheless, a significant amount is disposed of in landfills as general waste (Schieber et. al., 2001). According to a study, the aquatic ecology in Costa Rica is out of balance as a result of rejected bananas being dumped in the rivers. Banana biomass's higher carbohydrate content raised the oxygen demand in the rivers, which in turn caused a decline in aquatic life (Zhang et.al., 2005).

Figure 1: The Banana peel showing its parts

2.2.1 BANANA PEEL COMPOSITION

According to the findings from Mishra et al. (2023), Banana peels provide other plant micronutrients in varying amounts: calcium (0.18%), magnesium (0.08%), sodium (0.02%) and 5.7% potassium, making them a good source of fertilizer for crops that require large amounts of potassium, including bananas, tomatoes, and peppers. Potassium is present at about 200 mg per fruit in banana skins (Mishra et al., 2023). It also contains primary metabolites like 50% of dietary fiber (10-20% pectin, 6-12% lignin, cellulose 7-9%, 6-9% hemicellulose), 7% crude proteins, 10% crude fat, 3% starch, polyunsaturated fatty acids (linoleic acid, α-linoleic acid), essential amino acids (leucine, phenylalanine, threonine, and valine), micronutrients (calcium, iron, magnesium, potassium, zinc (Korni et. al., 2023).

2.3 BANANA FIBRE

Investigation and research into the use of natural plant fibers has been on in recent years. The fibers have been recognized as superior to synthetic ones as a result of their renewable source, lightweight composition, enhanced tensile and biochemical characteristics and biodegradable nature; due to the widespread use of bananas globally, banana fiber holds significant importance (Iqbal et. al., 2023, Dhivahar et al., 2025). Banana waste consists of high-quality fiber that possesses promising qualities for various industrial uses. They include the manufacture of reinforced composite materials for automobiles, textiles, pulp and paper, sanitary pads, food products, aerospace components, construction materials, and many other composite materials (Iqbal et. al., 2023).

### 3.0 Environmental Impacts and carbon footprints of Banana Waste

### Awareness of global warming and climate change has increased in the past decades, and led to demand in products with the least carbon footprint or global impact. Life cycle assessment and carbon footprint are two of the environmental indicators used to evaluate the overall environmental impact of the products. The entire product cycle from production to consumption is analysed by life cycle assessment; to understand the overall strategies of the product development to keep it environmentally friendly (Ounsaneha et. al., 2019). In a study by Roibás et. al., (2015), it was noted that the Ecuadorian banana had a carbon footprint of 1.28 tonnes of CO2 per tonne of banana. During banana production, waste disposal of the stems is one of the major contributors to global warming (Rattanapan, and Ounsaneha, 2022). The plantation stage recorded the highest impact, next is the transportation stage in Turkey (Yasin et. al., 2025). An average of ~114 million tonnes of banana waste is produced worldwide annually (Acevedo et. al., 2021). When banana waste is decomposed naturally by dumping in landfills, greenhouse gases are released (Housagul et. al., 2014).

4.0 ENVIRONMENTAL USES OF BANANA WASTES

When solid wastes are disposed and handled inappropriately, it causes pollution. Discarding of various types of wastes leads to all kinds of environmental pollution which includes air, soil and water pollution like clogging of water drains, stagnant water creation for different organisms like insects breading during summer season (Alam and Ahmade, 2013). Typically disposed of in municipal landfills, banana wastes contribute to environmental challenges. Yet, they can be put to many uses which offer promising potential for environmental sustainability. Harnessing these uses presents an opportunity to convert waste into economically and environmentally valuable processes, thereby addressing environmental concerns through sustainable innovation.

4.1 WASTEWATER TREATMENT

Adsorption is an interfacial phenomenon in which ions from solution adhere to solid material with which they come into contact, held by chemical and/or physical forces of attraction (Babel and Kurniawam 2003). Adsorption can be classified as physical adsorption or chemisorption. In physical adsorption, the adsorbate molecules physically bond to the surface of the adsorbent via weak van der Waals forces; this process is reversible due to low energy associated with it. Chemisorption occurs when adsorbate molecules interact with the adsorbent surface by chemical bonds (either covalently or ionically); high energy is released, and the process is irreversible because strong bonds are formed (semi-permanent (Somorjai, 1994).

Banana peel has been used as a constituent of bio adsorbents with high adsorption capacity for different types of contaminants due to the presence of hydroxyl, carboxyl, and amino groups (Munagapati et al., 2020); it has also been found to be effective in removing contaminants from wastewater including organic pollutants, dyes, and heavy metals. The removal of copper ions using the isolated antioxidant compounds from banana peels confirms that they can be used as an adsorbent (Fouda-Mbanga et al., 2024). Research has shown that it can remove heavy metals and organic chemicals from aqueous solutions. Natural or physically and chemically engineered banana peels are good adsorbent for removing these pollutants from aqueous solutions (Albarelli et.al., 2011).

Banana peel have also been used to purify carcinogenic materials from used frying oil, brightening the oil by adsorbing peroxides (Bhavani et al., 2023). In addition to their adsorptive abilities, banana peel fibers have also been incorporated in commercial spiral-wound reverse osmosis membranes (Moerz and Huber, 2015), while chemically modified banana peel has been used in the development of microporous/mesoporous carbon with intestine-like characteristics for size-selective separation of proteins and potential drug separation and biological macromolecule immobilization (Ali et. al., 2006).

# Over the past three decades, nearly all biological systems worldwide in wastewater, groundwater, surface water and sludge have contained pharmaceutical compounds (Caban and Stepnowski, 2021). The ecosystem is affected by a multitude of sources, as well as extensive use of therapeutic agents, making the problem of pharmaceutical pollution difficult to solve (Madhura 2019). In total, about 11,926 pharmaceutically active chemicals, including 713 in wastewater have been detected globally in wastewater effluents and classified as harmful chemicals to aquatic organisms under European regulations (Carolin et al., 2021). Al-sareji et al. (2024) demonstrated the synthesis of activated carbon from banana peels as an agro-industrial waste for removing pharmaceuticals: amoxicillin and carbamazepine from various water matrices. Banana peel activated carbon has been found to be a highly effective adsorbent for removal of amoxicillin and carbamazepine from different waters. It could potentially serve as an inexpensive alternative to expensive materials such as granular, ion-exchange resins, powdered activated carbons, carbon nanotubes, etc., in the treatment of liquid wastes containing organic micro pollutants.

# 4.2 USE AS BIO-FERTILIZER

# Banana peels are one of the main agricultural waste products with an annual production of 40 million tonnes, or approximately 35% of the weight of all bananas (Mishra et. al., 2023). The opportunities and challenges for the development of banana by-products include their use in sustainable food and non-food operations as thickeners, colorants and flavors, sources of macronutrients and micronutrients, natural cuticles, animal feed, natural fibers, and sources of naturally occurring bioactive composites and bio-fertilizers. Banana peel is a readily available organic waste material that contains various nutrients and other organic compounds that can be used as an effective biofertilizer. Recently, banana skin has been employed as a biofertilizer because it increases crop yields by 25-45% (Mishra et. al., 2023).

# In order to improve the soil's morphology, elevate its pH and supply nutrients, alkaline banana peel powder is utilized (Jabbour et. al., 2023, Khanyile et al., 2024)). Compared to chemical fertilizers and chicken manure, organic fertilizers produced from composted banana waste are thought to be more practical and reasonably priced (Jabbour et. al., 2023). The mortality rate of banana plantations was considerably decreased by solid-state fermentation of banana waste into a soil conditioner that promotes growth and recycling it as fertilizer for banana cultivation (Jabbour et. al., 2023).

The banana waste is suitable for anaerobic digestion in which the solid digestate has suitable nutrients to serve as fertilizer because it is organic. This approach also brings added benefits such as the production of high-quality fertilizer to support better farming practices and improved management of banana waste." (Oyaro et. al., 2020).

4.3 USE AS BIO-PLASTIC FILM

Plastics are one of the major contributors to land pollution. Biodegradable and biocompatible polymers have an increasing role in both basic research areas as well as applied research fields around the world. Bioplastics are plastics made from renewable biomass sources, such as agricultural byproducts and food waste. It is strong, flexible, transparent, a good barrier, heat resistant, and biodegradable. Jangra et al., (2023) reported successful production of bioplastic sheets from banana peel and industrial maize starch with 4% maize starch content that provided tensile strength with maximum value of 3.50 n/m2; water absorption test showed more than 50% increase in water intake for all the films, and all bioplastic films disintegrated within a week during a biodegradability test.

Plastics are preferred over metals, paper, and other materials due to their advantageous properties, including low weight, cost-effectiveness, and durability. Consequently, they are utilized across nearly all industrial sectors.  In 2017, global plastic consumption surpassed 300 million tons (Arjun et. al., 2023). However, this widespread usage has resulted in significant environmental concerns, with plastic waste accumulating globally, including in marine ecosystems. Furthermore, the plastic industry faces notable economic and environmental challenges.

In several countries, bioplastics are primarily used in products such as disposable cutlery, diapers, and packaging materials. It was projected that global bioplastic production in industrial applications would reach 7.8 million tons by 2019, highlighting the strong potential of this sector. Nevertheless, the cost of producing bioplastics from microbial sources remains relatively high compared to those derived from renewable raw materials. As a result, most manufacturers have shifted their focus toward production methods that utilize renewable resources (Akiran et al., 2019).

4.4 USE AS COMPOST

Composting is a biological process that facilitates the decomposition of solid organic waste through the activity of living organisms such as bacteria, fungi, worms, and insects. This transformation results in the production of organic compost, which, when integrated into soil, enhances the availability of macronutrients, secondary nutrients, and micronutrients essential for plant growth. Additionally, compost improves soil fertility and augments the porosity of clay-rich soils, thereby promoting better aeration and water retention (Birhanu and Berisa, 2015)

Banana peel compost serves as a valuable organic fertilizer that enhances soil fertility, supports plant nutrition, and promotes the growth of beneficial microorganisms crucial to root development and overall plant health. Additionally, its use contributes to environmental protection by transforming waste into productive agricultural input. Given that discarded banana peels may harbor harmful pathogens capable of causing diseases in humans, converting them into compost helps mitigate these health risks (Teshome, 2022). As highlighted by Alam and Ahmade (2013), studying the composting potential of banana peels offers significant benefits for both environmental sustainability and public health. Such research also provides essential direction for future community-based agricultural programs and scientific inquiries.

Figure 2: The many benefits of Banana peels.

4.5 USE FOR BIOGAS PRODUCTION

Relying on petroleum for power generation is not a sustainable solution due to the volatility of global crude oil prices and the negative climate effects caused by increased greenhouse gas emissions. Additionally, fossil fuels are non-renewable, their reserves are depleting, and their continued use could place a significant strain on foreign exchange in the future. Hence, utilizing agricultural waste—including banana waste—offers a viable alternative that can help address environmental, health, and energy challenges.

4.6 USE AS BIOSTIMULANTS IN BIOREMEDIATION

Bioremediation technology employs microorganisms in an effort to reduce, remove, contain, or alter contaminants in soils, sediments, water, and air. Bioremediation can be defined as the use of microorganisms to destroy or immobilize waste materials (Shanahan 2004) and occurs through mineralization, transformation, or alteration of the dangerous chemicals (Shannon and Unterman 1993).

Synthetic fertilizers are costly, however, they are a needed input for agriculture in developing countries such as Nigeria. These challenges increased the drive toward environmental sustainability to seek out organic substrates that can be used instead of synthetic fertilizers (Nwogu et al. (2015). In the soil and water environment, biodegradation of hydrocarbons by microorganisms is often limited because of nitrogen (N) and phosphorus (P) deficiency (Abioye et al. (2012).

Therefore, the addition of organic nutrients (biostimulation) or nitrogen-rich inorganic nutrients is another means to enhance bioremediation (Walworth et al., 2007). A variety of high-nitrogen organic substrates are being used as biostimulants; these include corn residues, sugarcane bagasse, yam and banana peels, sawdust, spent brewing grain, rice husks, coconut shells, cow, pig, and poultry manures (Agarry, S. E. and Jimodu 2013).

Natural bioremediation has been used for centuries in wastewater treatment by civilizations, but the deliberate use for reduction of hazardous wastes is a more recent development. According to Venosa et al, (2002), the most critical factor for oil spill bioremediation is the presence of microorganisms with the correct metabolic capabilities. These communities are adapted and become enriched with certain groups that display genetic change (Leahy and Colwell 1990). The adapted microbial community can respond to hydrocarbon pollutants in hours (Atlas and Bartha, 1998), show increased biodegradation rates compared to unexposed communities (Leahy and Colwell, 1990), and are considered good indicators of which microorganisms are the most active oil degraders of an environment. Therefore, isolation of high numbers of specific oil degrading microorganisms from an environment is often interpreted as evidence that those microorganisms are also the most active oil degraders of that environment and candidates for bioremediation of petroleum polluted sites.

Since crude oil is composed of a complex mixture of compounds and individual microorganisms can metabolize only a limited number of hydrocarbon substrates, degradation of petroleum hydrocarbons requires bacterial consortia capable of degrading a broader range of hydrocarbons (AL-Saleh 2009, Bordenave 2007). The rate of biodegradation depends on the availability of nutrients and other factors that promote biological functions.

4.7 USING BANANA PEELS TO REMOVE HEAVY METALS FROM ACID MINE DRAINAGE

Mining activities pollute water with heavy metals, which will continue to degrade water quality if remedial actions are not taken (Ashton et al., 2001). In fact, the degradation of water resources is a major environmental problem that the world faces today. The ecological damage caused by this degradation of water resources is a global issue and because water is an extremely important resource in the world, it needs to be protected. Although many treatment technologies exist for the removal of heavy metals from wastewater and acid mine drainage (AMD), most do not remove pollutants completely and some are too expensive with also significant disposal challenges (Hossain et al., 2012). Therefore, there is a need to develop a more effective and affordable technology for removing heavy metals from AMD. An alternative technique that is gaining popularity over recent years is adsorption using agricultural waste; banana peel waste in particular, which is a low cost environmental friendly bio-material that has been abandoned but has great potential because of its high content of cellulose, hemicellulose, lignin and carboxylic groups capable of adsorbing heavy metals from the solution to the surface.

Mahlangu et al. (2018) investigated the adsorption capacity of banana peels as an inexpensive adsorbent for heavy metals in synthetic water, actual acid mine drainage (AMD), and fortified AMD conditions. They concluded that banana peels can effectively serve as a low-cost material for removing zinc (Zn), copper (Cu), and manganese (Mn) from AMD.

Effects of banana peel compost rates on Swiss chard growth performance and yield in Shirka district, Oromia, Ethiopia Mahlangu et. al., 2018, explored the adsorption potential of banana peels as low cost adsorbent for heavy metals under synthetic water, actual AMD and fortified AMD conditions and concluded that banana peel can be used as a cheap adsorbent to remove the Zn, Cu and Mn under AMD conditions.

5.0 CONCLUSION

Recycling Banana peels is a great way to manage trash. The addition of waste value is also important in achieving the Sustainable Development Goals (SDG), particularly SDG 13, Climate Action. Also, while creating value-added products for soil amendment and material development, the valorization of banana-derived wastes aligns with circular economy principles by reducing environmental burden.  Further field-based and experimental studies are recommended to optimize application rates, processing methods, and long-term impacts on soil health and crop productivity, thereby facilitating the large-scale adoption of banana waste-based materials in sustainable agriculture and related industries.

REFERENCES

Alam, P. and Ahmade, K. (2013). Impact of solid waste on health and the environment. Int. J. Sustain. Dev. Green Econ. (IJSDGE) 2 (1), 165–168.

Afzal, M.F.; Khalid, W.; Akram, S.; Khalid, M.A.; Zubair, M.; Kauser, S.; Mohamedahmed, K.A.; Aziz, A.; Siddiqui, S.A. (2022). Bioactive profile and functional food applications of banana in food sectors and health: A review. Int. J. Food Prop. 25: 2286–2300. [Google Scholar] [CrossRef]

Albarelli, J.Q, Rabelo, R.B, Santos, D.T, Beppu, M.M, Meireles, M.A. (2011). Effects of supercritical carbon dioxide on waste banana peels for heavy metal removal. J Supercrit Fluids. 58: 343–351. doi: 10.1016/j.supflu.2011.07.014. [DOI] [Google Scholar

Alzate Acevedo, S.; Díaz Carrillo, Á..J.; Flórez-López, E.; Grande-Tovar, C.D. (2021). Recovery of banana waste-loss from production and processing: A contribution to a circular economy. Molecules 26: 5282. [Google Scholar] [CrossRef]

Ashton, P.J., Love, D., Mahachi, H. and Dirks, P., (2001). An overview of the impact of mining and mineral processing operations on water resources and water quality in the Zambezi, Limpopo and Olifants Catchments in Southern Africa. Contract Report to the Mining, Minerals and Sustainable Development (Southern Africa) Project, by CSIR-Environmentek, Pretoria and Geology Department, University of Zimbabwe Harare.Report No.ENV-PC, 42.

Abioye, O. P., Abdul A. R., Agamutha, P. (2012): Biodegradation of used lubricating oil by microbes isolated from pristine soil environment. Journal of science. Pp 1-7

Ali, M., Qamhiyah, A., Flugrad, D., Shakoor, M. (2008). Theoretical and finite element study of a compact energy absorber. Adv Eng Softw.39: 95–106. doi:10.1016/j.advengsoft.2006.12.006. [DOI] [Google Scholar]

AL-Saleh, H. D. and Obuekwe, C. (2009). “Predominant culturable crude oil-degrading bacteria in the coast of Kuwait,” International Biodeterioration and Biodegradation, 63: 400-406.

Agarry, S. E. and Jimodu, L. A. (2013): Application of Carbon-Nitrogen supplementation from plant and animals sources in situ soil bioremediation of diesel oil. Journal of environment and earth science. Pp 51-62

Al-sareji, O.J.; Grmasha, R.A.; Meiczinger, M.; Al-Juboori, R.A.; Somogyi, V.; Hashim, K.S. (2024). A Sustainable Banana Peel Activated Carbon for Removing Pharmaceutical Pollutants from Different Waters: Production, Characterization, and Application. Materials 17: 1032. https://doi.org/10.3390/ma17051032

Arjun, J., Manju, R., Rajeswaran, S.R. and Chandhru, M. (2023). Banana peel starch to biodegradable alternative products for commercial plastics Department of Microbiology, Hindusthan college of Arts and Science, Coimbatore, Tamil Nadu, India. GSC Biological and Pharmaceutical Sciences, 22: 234–244.

Bhavani, M., Morya, S., Saxena, D., & Awuchi, C. G. (2023). Bioactive, antioxidant, industrial, and nutraceutical applications of banana peel. International Journal of Food Properties, 26(1), 1277–1289. https://doi.org/10.1080/10942912.2023.2209701

Bordenave, M. S. Goñi-Urriza, P. Caumette, and R. Duran, (2007) “Effects of heavy fuel oil on the bacterial community structure of a pristine microbial mat,” Applied and Environmental Microbiology, 73: 6089-6097.

Birhanu, Y., Berisa, G., (2015). Assessment of solid waste management practices and the role of public participation in Jigjiga Town, Somali Regional State, Ethiopia. Int. J. Environ. Protect. Pol. 3: 153–168.

Caban, M.; Stepnowski, P. (2021). How to decrease pharmaceuticals in the environment? A review. Environ. Chem. Lett. 19: 3115–3138. [Google Scholar] [CrossRef]

Dhivahar, P., Srinithi, A., & Devaki, E. (2025). Applications of banana fiber – A general overview. International Journal for Research in Applied Science and Engineering Technology, 13(1), 1021–1027.

Femina, Carolin, C.; Senthil Kumar, P.; Janet Joshiba, G.; Vinoth Kumar, V. (2021). Analysis and removal of pharmaceutical residues from wastewater using membrane bioreactors: A review. Environ. Chem. Lett. 19: 329–343.

Fouda-Mbanga, B. G., Velempini, T., Pillay, K., & Tywabi-Ngeva, Z. (2024). Heavy metals removals from wastewater and reuse of the metal loaded adsorbents in various applications: A review. Hybrid Advances, 6, 100193. https://doi.org/10.1016/j.hybadv.2024.100193

Ganesh, K.S., Sridhar, A. and Vishali, S. (2021). Utilization of fruit and vegetable waste to produce value-added products: Conventional utilization and emerging opportunities- A Review. Journal pre-proof.

Housagul, S.; Sirisukpoka, U.; Boonyawanich, S.; Pisutpaisal, N. (2014). Biomethane production from co-digestion of banana peel and waste glycerol. Energy Procedia. 61: 2219–2223. [Google Scholar] [CrossRef]

Hassan, H. F., Hassan, U. F., Usher, O. A., Ibrahim, A. B. (2018). Exploring the Potentials of Banana (Musa Sapietum) Peels in Feed Formulation. International Journal of Advanced Research in Chemical Science (IJARCS). 5: 10-14.

Housagul, S. Sirisukpoka, U. Boonyawanich, S and Pisutpaisal, N. (2014). Biomethane production from co-digestion of banana peel and waste glycerol,‖ Energy Procedia, 61: 2219-23.

Prospect of waste banana fiber use in industry: A narrative review. Farhana Iqbal, S. M Humaun Kabir, Md. Iftakharul Muhib, Md. and Mehadi Hasan Khan Rupok, Md. (2023). International Journal of Science and Research Archive, 10: 828–839.

Hatzikiseyian, A. (2017): Principles of bioremediation processes. Trends in bioremediation and phytoremediation. Pg 23

Hikal, W. M. (2022). Banana peels: A waste treasure for human being. Journal of Food Quality and Hazards Control, 9(2), 123–131.

Hossain, M., Ngo, H.H., Guo, W. and Nguyen, T., (2012). Removal of copper from water by adsorption onto banana peel as bioadsorbent. International Journal of Geomate, 2: 227-234.

Jos, A.; Repetto, G.; Rios, J.C.; Hazen, M.J.; Molero, M.L.; Del Peso, A.; Salguero, M.; Fernández-Freire, P.; Pérez-Martín, J.M.; Cameán, A. (2003). Ecotoxicological evaluation of carbamazepine using six different model systems with eighteen endpoints. Toxicol. Vitro. 17: 525–532.

Khanyile, N., Dlamini, N., Masenya, A., Madlala, N. C., & Shezi, S. (2024). Preparation of biofertilizers from banana peels: Their impact on soil and crop enhancement. Agriculture, 14(11), 1894. https://doi.org/10.3390/agriculture14111894

Leahy, J.G.; Colwell, R.R. (1990) Microbial Degradation of hydrocarbons in the environment. Microbial Reviews, 53: 305- 315.

Munagapati, V.S, Wen, J.C, Pan, C.L, Gutha, Y., Wen, J.H, Reddy, G.M. (2020). Adsorptive removal of anionic dye (Reactive Black 5) from aqueous solution using chemically modified banana peel powder: kinetic, isotherm, thermodynamic, and reusability studies. Int J Phytoremediat. 22: 267–278. doi: 10.1080/15226514.2019.1658709. [DOI] [PubMed] [Google Scholar]

Manzoor, A.; Ahmad, S. (2021). Banana peel: Characteristics and consideration of its extract for use in meat products preservation: A review. ACS Food Sci. Technol. 1: 1492–1506. [Google Scholar] [CrossRef]

Madhura, L.; Singh, S.; Kanchi, S.; Sabela, M.; Bisetty, K.; Inamuddin. (2019). Nanotechnology-based water quality management for wastewater treatment. Environ. Chem. Lett. 17: 65–121.

Jabbour, M.Y., Souied, L.K. Baltagi, T.K., Sheet, I.H. and Sakr, S.S. (2023). Banana Peel Utiliza tion: Practice and Perspective, Highlights from Lebanon. IJCMCR. 34(4): 002

Moerz, S.T, Huber, P.  (2015). pH-dependent selective protein adsorption into mesoporous silica. J Phys Chem C. 119: 27072–27079. doi: 10.1021/acs.jpcc.5b09606. [DOI] [Google Scholar]

Mahlangu, J.M., Simate, G.S., de Beer, M. (2018). Adsorption of Mn2+ from the Acid Mine Drainage using banana peels. Intl J Water Wastewater Treat 4(1): dx.doi.org/10.16966/2381-5299.153

Nwogu, T. P., Azubuike, C. C., Ogugbue, C. J. (2015): Enhanced bioremediation of soil artificially contaminated with petroleum hydrocarbon after amendment with Capra aegagrushircus (goat) manure. International journal of biotechnology research. Pp 1-7.

Narwal, P.; Kapoor, B.; Negi, N.P. (2024). Decoding the chemical repertoire, antimicrobial synergy, and antioxidant mastery of banana pulp and peel extracts. J. Plant Biochem. Biotechnol. 33: 436–446. [Google Scholar] [CrossRef]

Ounsaneha, W.; Buadit, T.; Rattanapan, C. (2019).  Assessment of human health impact based on life cycle assessment: A case study of Thai retread tire. IOP Conf. Ser. Mat. Sci. Eng. 773, 012038. [Google Scholar] [CrossRef]

Oyaro, D.K., Oonge, Z.I. and Odira, P.M. (2020). Anaerobic Digestion of Banana Wastes for Biogas Production. Journal of Civil and Environmental Engineering, 10:3

Provin, A.P.; Medeiros d’Alva, A.; de Aguiar Dutra, A.R.; Salgueirinho Osório de Andrade Guerra, J.B.; Leal Vieira Cubas, A. (2024). Closing the cycle: Circular economy strategies for the textile industry using banana farming waste. J. Clean. Prod. 470: 143352. [Google Scholar] [CrossRef]

Roibás, L.; Elbehri, A.; Hospido, A. (2015). Evaluating the sustainability of Ecuadorian bananas: Carbon footprint, water usage and wealth distribution along the supply chain. Sustain. Prod. Consum. 2: 3–16. [Google Scholar] [CrossRef]

Rattanapan, C and Ounsaneha, W. (2022). The Potential for Value-Added Banana Production Waste under Circular Economy Concept. International Journal of Environmental Science and Development, 13

Korni, R.D., Boddepalli, T., Elusuri, J. and Panda, J. (2023). Banana Peel: A potential waste product with numerous pharmacological activities GSC Biological and Pharmaceutical Sciences, 23: 160–174.

Rattanapan, C. and Ounsaneha, W. (2022). The potential for value-added banana production waste under circular economy concept. Int. J. Environ. Sci. 13: 92–96. [Google Scholar] [CrossRef]

Mishra, S., Kesharwani, S., Chivate, M., Chauhan, A.K. and Mishra, V. (2023).   Development of Novel Biofertilizer by Combining the Banana Waste and Cyanobacteria in Sustainable Agriculture. Int. J. Food Ferment. Technol., 13: 69-86.

Jangra, M.R., Nehra, K.S., Deepika, Sanju, Ritu, Rani, S., Sharma, A., Dhankhar, K., and Rani, P. (2023), Utilisation of Waste Banana Peels for the Preparation of Environment Friendly Bio-plastic Film: A Preliminary Study. Shodh Sari-An International Multidisciplinary Journal .   2: 324-333.

Somorjai, G.A., (1994). The surface science of heterogeneous catalysis. Surface science, 299, pp 849-866.

Schieber, A.; Stintzing, F.C.; Carle, R. (2001). By-products of plant food processing as a source of functional compounds—Recent developments. Trends Food Sci. Technol. 12: 401–413. [Google Scholar] [CrossRef

Shaukat N.; Farooq, U.; Akram, K.; Shafi, A.; Hayat, Z.; Naz, A.; Hakim, A.; Hayat, K.; Naseem, S.; Khan, M.Z. (2023). Antimicrobial potential of banana peel: A natural preservative to improve food safety. Asian J. Agric. Biol. 3:1–6. [Google Scholar] [CrossRef]

Shanahan, P. (2004) Bioremediation. Waste Containment and Remediation Technology, Spring 2004, Massachusetts Institute of Technology, MIT OpenCourseWare http://ocw.mit.edu/NR/rdonlyres/Civil-and-EnvironmentalEngineering/1-34Spring2004/335613D5-6D6F-413F-9098- 453E8AC20BC2/0/lecture12.pdf

Shannon, M.J. and Unterman, R. (1993) Evaluating bioremediation: distinguishing fact from fiction.Annual Review of Microbiology v47.(Annual 1993): pp715(24)

Sagar, N.A., Pareek, S., Sharma, S., Yahia, E.M. and Lobo., M.G. (2018). ―Fruit and vegetable waste: Bioactive compounds, their extraction, and possible utilization,‖ Comprehensive Reviews in Food Science and Food Safety, 17: 512-31.

Tavalı, İ. E. (2022). Monitoring the short-term effect of banana waste compost on the enzymatic activities of soil associated with nitrogen and phosphorus. International Journal of Agricultural and Wildlife Sciences, 8(2), 328–339. doi:10.24180/ijaws.1103908

Thangavelu, R., Saraswathi, M.S., Uma, S., Loganathan, M., Backiyarani, S. Durai, P., Raj, E.E., Marimuthu, N., Kannan, G and Swennen, R (2021). ―Identification of sources resistant to a virulent Fusarium wilt strain (VCG 0124) infecting Cavendish bananas,‖ Scientific Reports,11: 3183.

Tewari, S.; Reshamwala, S.M.S.; Bhatt, L.; Kale, R.D. (2024). Vegan leather: A sustainable reality or a marketing gimmick? Environ. Sci. Pollut. Res. 31: 3361–3375. [Google Scholar] [CrossRef]

Vu, H.T.; Scarlett, C.J.; Vuong, Q.V.  (2017). Optimization of ultrasound-assisted extraction conditions for recovery of phenolic compounds and antioxidant capacity from banana (Musa cavendish) peel. J. Food Process. 41, e13148. [Google Scholar] [CrossRef]

Venosa, A. D., Lee, K., Suidan, M. T., Garcia-Blanco, S., Cobanli, S., Moteleb, M., Haines, J.R., Tremblay, G., and Hazelwood, M. (2002) Bioremediation and biorestoration of a crude oilcontaminated freshwater wetland on the St. Lawrence River. Bioremediation Journal, 6 (3).

Weshah, S.F., Al-Hafud, A.S. (2023). The Effect of Replacing Wheat Flour with Mango Peel Powder and Studying its Chemical and Microbial Properties in the Laboratory Cake. In IOP Conference Series: Earth and Environmental Science. IOP Publishing, 1158: s112005.

Wafaa, M.H., Hussein, A.H.S.A., Amra, B., Kirill, G.T, Javed, S., Miroslava, K., Mohammed, E., Maria, A. (2022). Banana Peels: A waste treasure for human being. Evidence-Based Complementary and Alternative Medicine. 2022; 1-9.

Walworth, J., Pond, A., Snape, I., Rayner, J., Ferguson, S., Harvey, P. (2007): Nitrogen requirement for maximizing petroleum bioremediation in a sub-antartic soil. Cold region science and technology. Pp 84-91

Teshome, Z.T. (2022).  Effects of banana peel compost rates on Swiss chard growth performance and yield in Shirka district, Oromia, Ethiopia.  Heliyon, 8:1-6.

# Yasin, M., Gangan, S and Panchal, S.K. (2025).  Banana Peels: A Genuine Waste or a Wonderful Opportunity?   Appl. Sci. 15: 3195.

Zhang, P.; Whistler, R.L.; BeMiller, J.N.; Hamaker, B.R. (2005). Banana starch: Production, physicochemical properties, and digestibility—A review. Carbohydr. Polym. (2005). 59: 443–458. [Google Scholar] [CrossRef]

Zhang, Y.; Geißen, S.U.; Gal, C. (2008). Carbamazepine and diclofenac: Removal in wastewater treatment plants and occurrence in water bodies. Chemosphere, 73:  1151–1161.