Original Research Article

Assessment of the physicochemical parameters of Okpoka Creek: a Tributary of Bonny River

.

ABSTRACT

Aim: This study investigated physico-chemical analysis across six major stations in Okpoka creek in the Niger Delta region of Nigeria (Oba, Abuloma, Kalio, Georgetown, Okrika, and Ogoloma). Study Design: Cross-sectional study Place and Duration of Study: Okpoka Creek, Abuloma Area, Kalio town, George Town, ATC Okrika and Ogoloma Area, between April 2021 and March 2022. Methodology: The study was carried out in Okpoka creek located in Okirika Local Government Areas. This study obtained samples from six distinct communities: Oba Town Area, Kalio Town Area, George Town Area, Ogoloma Town Area, ATC Okrika Area, and Abuloma Town Area, all situated in Okrika Local Government Area, Rivers State, Nigeria, during both dry and rainy seasons.Water samples from various locations were collected throughout a 12-month period (April 2021 March 2022). On reaching the laboratory, the specimens from each sampled station were analyzed for physcochemical parameters using established analytical techniques. Data generated from this study were collated and subjected to analysis of variance using SPSS version 23. Results: The results showed that water quality assessments revealed significant spatial and seasonal variations in key parameters. Dissolved oxygen levels (7.33-7.68 mg/L) remained within acceptable ranges, though lower values in Kalio and George Town suggested organic loading. Notable pollution hotspots were identified, including elevated phosphate concentrations in Oba Town (5.51 mg/L) and high nitrate levels in ATC Okrika (3.25 mg/L), indicative of agricultural runoff. Ogoloma River showed exceptional salinity (38.69 ppt) and conductivity (37.02 mS/cm), likely due to saltwater intrusion. Seasonal patterns demonstrated reduced oxygen in wet months (7.30 mg/L in July-August) and increased turbidity in dry seasons (6.06 NTU), reflecting the region's tropical hydrological dynamics Conclusion: Based on the findings, to maximize fisheries potential and preserve the best physiological condition of fish species, the physico-chemical parameters of surface waters at the landing sites should be continuously monitored to prevent further deterioration.

Keywords: physico-chemical parameters, Okpoka Creek, Bonny River

1. INTRODUCTION

The Okpoka creek is a critical ecological and economic resource, supporting rich biodiversity and providing livelihoods for communities dependent on its fisheries. However, increasing anthropogenic pressures such as oil exploration, pollution, overfishing, and habitat degradation have raised concerns about the sustainability of its fish populations. Research on the population dynamics of fishes in the Bonny River is necessary to provide scientific data for sustainable management of its fisheries resources. Physico-chemical parameters constitute important abiotic components of aquatic ecosystems that influence species composition, diversity, stability, abundance, productivity, migration, biodiversity and physiological condition of aquatic organisms [1,2]. These parameters are used to detect any perturbation in the aquatic environment. Temperature is one of the most important physical characteristics of the ecosystem. It affects several water quality parameters that are of concern for domestic, environmental, industrial and agricultural applications [3]. The chemical and biological reaction rates of water increase with increases in water temperature [3,4]. Water temperature plays an important role in governing the growth of organisms [5] and temperatures in tropics vary between 21ºC and 32ºC [6,7]

Dissolved oxygen is one of the most important parameters of water as its correlation with water body gives direct and indirect information on the status of the water e.g. bacterial activities, photosynthesis, availability of nutrients, stratification etc [8]. The concentration of dissolved oxygen may not be a major limiting factor for Nile Tilapia as they can tolerate levels as low as 3 – 4 mg/l [9]. In rivers and streams, turbulence ensures that oxygen is uniformly distributed across the water and in very shallow streams the water may be super saturated [10]. Dissolved oxygen is an important environmental parameter that decides ecological health of a stream and protects aquatic life [11].

The biological oxygen demand of water is the amount of dissolved oxygen taken up by bacteria in degrading oxidizable matter in the sample, measured after 5 days. The biological oxygen demand is simply the amount by which the dissolved oxygen level drops during the incubation period and gives a direct measure of primary productivity [12]. Samples with high values have direct health implications [12], but it is an important indicator of overall water quality. Biological oxygen demand is the amount of oxygen utilized by microorganisms in stabilizing organic matter. According to WHO [13] and EPA [12], the stipulated guidelines are 5 and ≤ 5 respectively. Conductivity is a measure of the ability of water to conduct electrical current. Conductivity provides good indication of the changes in water composition, particularly its mineral concentration. Variations of dissolved solids in water could affect the relative quantities of the various components. There is a relationship between conductivity and total dissolved solids in water. As more dissolved solids are added, water’s conductivity increases [10]. Conductivity of salt water is usually higher than that of freshwater because the former contains more electrically charged ions than the latter. Moreover, conductivity is an index of the total ionic content of water and, therefore, indicates freshness or otherwise of the water [14]. Conductivity of freshwater varies between 50 and 1500 mhos/cm [9], but some polluted waters reach 10,000 hs/cm. Seawater has conductivity around 35,000 mhos/cm and above. Hydrogen Ion Concentration (pH) is a measure of whether the water bodies are acidic or alkaline. pH is known to influence the physiological functions of fish and other aquatic lives. The pH of water is important because many biological activities can occur only within a narrow range [15]. Thus, pH range for diverse fish production is between 6.5 – 9. Any variation beyond acceptable rate range could be fatal to many aquatic organisms.

A maximum value of 400 mg/l of total dissolved solids is permissible for diverse fish production. In natural water, dissolved solids are composed mainly of solutes like carbonates, chlorides, iron, manganese sodium, calcium etc. There is a relationship between conductivity and total dissolved solids in water. As more dissolved solids are added water’s conductivity increases [9]. Variation of dissolved solid in water could affect the relative quantities of the various components. Untreated storm water runoff from urban areas can contain levels of some parameters (e.g. total dissolved solids) that exceed those found in untreated wastewater [16]. Salinity of estuaries usually increases away from a freshwater source such as river, although evaporation sometimes causes the salinity at the head of the estuary to exceed seawater. Salinity is one of the major factors influencing algal zonation and distribution within the estuaries both in terms of range of values and rate of change [17]. Chindah [17] observed that periphyton community responded to salinity gradient in a tropical estuary, Niger Delta. Phosphate values in surface waters are generally observed to be low [18]. Phosphate ions are not desirable in surface water, because they are the most important growth limiting factor in eutrophication and result in a variety of adverse ecological effects 19]. Several authors have reported that inorganic fertilizers are washed into standing water bodies from the farms that surround them [20]. Other forms of nitrogen present in natural waters include molecular nitrogen (N2) in solution, ammonia as NH3, ammonium and ammonia hydroxides (NH4 and NH4OH) and nitrites NO2 [21]. Nitrates and phosphates are good indicators of eutrophication. This study investigated physico-chemical analysis across six major Rivers in the Niger Delta region of Nigeria (Oba, Abuloma, Kalio, Georgetown, Okrika, and Ogoloma).

2. materialS and methods

2.1 Study Area

The study was carried out in Okpoka creek located in Okirika Local Government Areas. It forms part of the Bonny River system, which flows into the Atlantic Ocean. This river runs through a network of mangroves, wetlands, and estuaries, which are integral to the region’s hydrology. The Okpoka creek feeds into the Bonny River, which acts as a major channel for shipping, especially for the oil and gas sector. Basin is characterized by low-lying plains, swampy terrains, and an intricate network of waterways. These features make it a flood-prone area and have brackish water, influenced by both tidal movements and freshwater inflow from inland Rivers.

2.2 Sampling Stations

This study obtained samples from six distinct communities: Oba Town Area, Kalio Town Area, Geoge Town Area, Ogoloma Town Area, ATC Okrika Area, and Abuloma Town Area, all situated in Okrika Local Government Area, Rivers State, Nigeria, during both dry and rainy seasons. The sample points' locations were determined using a Global Positioning System (GPS). The sampling stations are illustrated in Figure 1, and the coordinates of the sample points are presented in Table 1.

Table 1: Coordinates of the Study Area (Okpoka Creek)

| Location | Coordinates |
| --- | --- |
| Oba Town Area | N 040 46’ 33.64” E0070 04’ 08.33” |
| Aboluma Area | N 040 46’ 13.0” E0070 03’ 52.1” |
| Kalio Town Area | N 040 45’ 41.4” E0070 04’ 03.2” |
| George Town Area | N 040 45’ 16.0” E0070 04’ 12.1” |
| ATC Okrika Area | N 040 44’ 40.7” E0070 04’ 34.8” |
| Ogoloma Area | N 040 43’ 55.8” E0070 04’ 44.2” |

Fig. 1: Map showing Okpoka Creek with Sampling Locations

2.3 Analysis of Physico-Chemical Parameters

2.3.1 pH

The pH measurements were conducted using a Hanna HI 98129 pH meter. The pH meter was calibrated with standard buffer solutions (pH 4.0, 7.0, and 10.0) before each measurement session to ensure precise results. Each replicate sample was analyzed, and the pH values were recorded to assess the water quality across the sampling stations.

.

2.3.2 Total Dissolved Solids (TDS), Electrical Conductivity (EC) Temperature and Dissolved Oxygen (DO)

The in-situ analysis of surface water at the landing sites involved measuring total dissolved solids, electrical conductivity, temperature, and dissolved oxygen values. This was achieved by immersing the instrument's probe into the water up to the specified point (3.5cm) on the meter for a duration of two (2) minutes. measuring instrument (Hanna HI 98194) was gently agitated at intervals to facilitate the probe's reading of total dissolved solids (TDS), electrical conductivity (EC), temperature, and dissolved oxygen (DO). Readings were recorded once the displaced value remained stable for a few seconds.

2.3.3 Biochemical Oxygen Demand (BOD) (mg/L)

BOD samples were using 250ml BOD glass bottles, and at each sampling station, the bottles were immersed in water to collect the samples. The utmost care was taken by allowing the BOD glass bottles to submerge in the water below the surface, collecting water samples before covering them with aluminum foil without fixation. Subsequently, the water samples were transported to the laboratory, where they were kept for 5 days to incubate at 20℃ before fixation. The samples were fixed using Winkler (3) Conc. H2+SO4- and titrated in the laboratory following the Winkler method outlined in the American Public Health Association, APHA [22] guidelines, with the azide modification method. This entire process was carried out in triplicates.

2.3.4 Salinity (ppt)

In situ measurements of salinity in parts per thousand (% or ppt) were conducted using a pen-type meter (model). After the meter reading stabilized, the instrument's probe was immersed at a depth of 5 to 10 cm, and the meter readings were recorded. The tests were performed in triplicates, and the mean value was computed.

2.3.5 Nitrate (mg/l)

A clear 1-liter plastic bottle was employed to collect and transport three identical samples of surface water for nitrate analysis, adhering to the APHA's [22] guidelines. The laboratory assessment of nitrate utilized the brucine method, relying on the principle that, under elevated temperatures and acidic conditions, brucine reacts with nitrate to generate a yellow color. In the analytical process, 10 mm of the water sample was placed in a test tube, and sulfuric acid was gradually added. After cooling for twenty minutes in a water bath, 0.2 ml of brucine sulfate was introduced and thoroughly mixed. The sample was then boiled in a water bath for 25 minutes, removed, and allowed to cool in a cool bath. Following this, the sample was transferred to a cuvette with a 4 ml diameter, and readings were taken at 410 mm using a Spectrophotometer model 21D. A blank sample of distilled water functioned as the reference reading for comparison, and the procedure was repeated using that sample.

2.3.6 Phosphate (mg/l)

Duplicate water samples were in a 1-liter plastic container for the analysis of phosphate, adhering to the APHA [22] guidelines. In the laboratory, the samples were analyzed using the Stannous chloride method, wherein a molybdate complex forms upon the reaction of ammonium and phosphate ions. Stannous chloride subsequently changed the molybdate in the complex into a blue color. A Spectrophotometer was utilized to quantify the phosphate responsible for the color in the sample. For the analysis, 2 ml of molybdate was introduced into a volumetric flask containing 50ml of treatment water. After determining the phosphate content with the Spectrometer Model 121D, 0.2 ml of Stannous reagent was added, and the mixture was thoroughly mixed. After a 10-minute period, 4ml of the treated water sample was transferred to a cuvette, and measurements were taken using a Spectrometer 121D at a wavelength of 690 mm. The sample procedure was duplicated to assess blank samples of ionized water.

2.4 Statistical Analysis

Data generated from this study were collated and subjected to analysis of variance using SPSS version 23. The graphs were plotted using Microsoft Excel version 2016

3. results and discussion

Table 2: Physico-chemical Properties based on Locations/Spatial differences in Physico-chemical Parameter

| Locations | pH | DO (mg/l) | Conductivity(ms/cm) | TDS (mg/l) | Salinity(ppt) | turbidity (mg/l) | Temp (oC) | NO3 (mg/l) | PO4(mg/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Oba Town | 7.86±0.18ab | 7.68±0.05a | 33.28±0.84a | 16.22±0.40a | 19.50±0.55a | 7.26±0.77a | 28.50±0.26a | 2.46±0.10ab | 5.51±0.51a |
| Abuloma | 8.00±0.08ab | 7.37±0.07be | 33.95±0.86a | 16.81±0.29a | 20.89±0.40a | 8.10±0.93a | 28.82±0.17a | 2.15±0.01ab | 4.66±0.11ac |
| KalioTown | 8.25±0.14a | 7.33±0.06ce | 34.10±0.71a | 16.93±0.25a | 20.54±0.42a | 4.60±0.20b | 28.86±0.18a | 1.69±0.05a | 3.15±0.18b |
| GeorgeTown | 7.72±0.11b | 7.37±0.07de | 36.08±1.06a | 17.94±0.36b | 22.85±0.50a | 4.29±0.14b | 28.63±0.26a | 1.26±0.15a | 4.17±0.37ac |
| ATC Okrika | 8.09±0.03ab | 7.42±0.09ae | 36.45±1.14a | 18.57±0.35b | 23.09±0.64a | 2.98±0.20b | 28.68±0.31a | 3.25±0.63b | 3.09±0.33bc |
| Ogoloma | 8.16±0.03ab | 7.50±0.08ae | 37.02±1.16a | 19.03±0.31b | 38.69±14.30a | 3.99±0.31b | 28.99±0.18a | 2.67±0.17ab | 4.61±0.51ac |

Key: mean ± SE of Physico-chemical parameters with the same alphabetic superscripts in the same column are not significantly different from each other (p>0.05).

Fig 2 : Mean concentration of physico-chemical parameters

Figure 3: Seasonal Variation of Physico-Chemical Parameter

Table 3: Seasonal Variation of the Physico-Chemical Parameters

| Months | pH | DO | Conductivity | TDS | Salinity (0/100) | turbidity | Temp | NO3 | PO4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Jan | 8.19±0.25a | 7.36±0.05a | 37.26±0.93a | 18.59±0.43a | 22.90±0.73a | 5.89±1.20a | 27.77±0.24a | 3.01±0.08a | 4.94±0.35ac |
| Feb | 8.18±0.23a | 7.32±0.07a | 37.29±0.96a | 18.59±0.43a | 23.08±0.79a | 6.06±1.24a | 27.78±0.23a | 1.19±0.11b | 3.51±0.51a |
| Mar | 8.20±0.25a | 7.34±0.07a | 37.31±0.94a | 18.60±0.42a | 23.03±0.79a | 6.05±1.32a | 27.78±0.24a | 2.61±0.06ab | 5.66±0.11bc |
| Apr | 7.98±0.01a | 7.93±0.02bc | 35.33±0.76ac | 16.99±0.61ac | 21.37±1.08a | 2.75±0.40a | 29.48±0.05b | 1.68±0.04b | 3.15±0.18a |
| May | 7.84±0.06a | 7.77±0.04bd | 35.05±0.82ad | 16.79±0.58ad | 21.30±1.06a | 2.68±0.53a | 29.44±0.05b | 1.98±0.13b | 5.17±0.37bc |
| Jun | 7.87±0.11a | 7.39±0.10ad | 36.37±1.02ae | 18.17±0.52a | 22.82±0.74a | 5.62±1.21a | 29.24±0.02b | 2.31±0.05b | 4.09±0.33ac |
| Jul | 8.10±0.07a | 7.30±0.01a | 31.81±1.14bcdef | 15.86±0.57bcd | 19.66±0.76a | 5.58±0.34a | 27.64±0.06a | 1.77±0.01b | 3.61±0.51a |
| Aug | 7.75±0.20a | 7.30±0.04a | 28.21±0.23b | 16.51±0.26a | 20.04±0.94a | 5.20±0.96a | 29.04±0.18b | 3.87±0.03a | 4.88±0.71ac |
| Sep | 7.86±0.13a | 7.58±0.17acd | 33.85±2.08af | 16.12±0.39bcd | 48.30±29.54b | 5.07±0.87a | 29.00±0.19b | 2.05±0.07b | 5.23±0.18bc |
| Oct | 8.27±0.24a | 7.34±0.10a | 36.44±1.00af | 18.25±0.54a | 22.87±0.75a | 5.72±1.21a | 29.32±0.03b | 2.33±0.11b | 4.01±0.52ac |
| Nov | 7.97±0.11a | 7.38±0.09ad | 36.40±1.02af | 18.31±0.50a | 22.89±0.74a | 5.92±1.10a | 29.27±0.02b | 1.80±0.09b | 3.84±0.49a |
| Dec | 7.95±0.13a | 7.37±0.10a | 36.45±1.02af | 18.24±0.54a | 22.86±0.72a | 5.90±1.07a | 29.22±0.02b | 2.28±0.12b | 3.74±0.55a |

Key: mean±SE of Physico-chemical parameters with the same alphabetic superscripts in the same column are not significantly different from each other (p>0.05).

Table 4: Multivariate matrix correlation between physico-chemical parameters

|  | pH | DO | Cond. | TDS | Salinity | Turbidity | Temp | NO3 | PO4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| pH | 1.00 |  |  |  |  |  |  |  |  |
| DO | -0.17 | 1.00 |  |  |  |  |  |  |  |
| Cond. | 0.09 | -0.15 | 1.00 |  |  |  |  |  |  |
| TDS | 0.22 | -0.22 | 0.72 | 1.00 |  |  |  |  |  |
| Salinity | 0.08 | 0.30 | -0.13 | 0.12 | 1.00 |  |  |  |  |
| Turbidity | -0.02 | -0.14 | -0.18 | -0.22 | -0.05 | 1.00 |  |  |  |
| Temp | -0.10 | 0.34 | -0.12 | -0.07 | 0.10 | -0.25 | 1.00 |  |  |
| NO3 | 0.09 | 0.25 | -0.21 | -0.13 | 0.07 | 0.69 | 0.69 | 1.00 |  |
| PO4 | 0.07 | 0.20 | 0.19 | 0.06 | 0.16 | 0.19 | 0.39 | 0.50 | 1 |

Keys: *= significant at p<0.05

The mean pH values ranged from 7.72 in Georgetown River to 8.25 in Kalio River. This values range is ideal for fish health and aquatic life. These values differed from the 6.6-7.9. These values fall within the acceptable WHO range of 6.5-8.5. pH is a crucial parameter for assessing water quality due to its impact on the chemical and biological processes that occur in water bodies. The pH of water can be influenced by natural and human-made factors, including atmospheric deposition, industrial activities, and agricultural practices. pH is an important parameter for assessing environmental sustainability. Changes in pH can affect the chemistry and biology of water bodies, leading to changes in ecosystem structure and function. Acidification of water bodies can lead to the loss of biodiversity, while alkalization can lead to the proliferation of harmful algal blooms. Changes in pH can also affect the productivity of aquatic ecosystems, which can have implications for food security and economic sustainability. As indicated by Environmental Protection Agency [23], the dominant part of aquatic organisms does better in the ideal range, however some can live in water with pH levels outside of this bracket. There are some components that can influence water pH, both regular and man-made. A greater part of normal changes happens because of contact with encompassing rock (especially carbonate containing ones) and different materials. Water pH can likewise change with precipitation (particularly acidic downpour) and wastewater or mechanical releases. Other anthropogenic activities that could influence pH are point source contamination, especially where there is poor buffering.

Results of the temperatures of the various rivers examined revealed a range of 27.64-29.48oC. This was similar to the ranges of Abowei [9], 27°C – 31°C; and 27°C – 31°C who carried out research in the Niger Delta too. Several factors have been shown to modify water temperature thereby causing a shift in the water temperature distribution, with an increase or decrease in temperature extremes: or a change in temperature variation. The effect of a change in water temperature may be direct, including thermal discharges; or indirect, land use changes, agricultural irrigation return-flows, flow modifications (river regulation), inter-basin water transfer, modification to riparian vegetation and global warming. The extent to which each of these thermal modifiers affects river systems in Nigeria is not known. Others include heated power station discharges, heated industrial discharges and returning water.

Salinity recorded an insignificantly different values between the study areas (p>0.05) being highest in Ogoloma River (38.69±14.30ppt) and least in Oba River (19.50±0.55ppt).

Dissolved oxygen (DO) value in the present study was highest in Oba River (7.68±0.05mg/l) and least in Kalio River (7.68±0.05mg/l) (p<0.05).

The mean conductivity values in the present study are between 33.28±0.84ms/cm (Oba River) to 37.02±1.16ms/cm (Ogoloma River). These values are lower than those reported by previous studies. The decrease in electrical conductivity in the present study could be due to lower salinity. Salinity is the main factor that influences electrical conductivity Environmental Protection Agency, [23]. Salinity has direct relationship with electrical conductivity. Temperature additionally influences conductivity by causing higher ionic activity and the dissolution of numerous salts and minerals University of Virginia Physics Department, reported conductivity as essential since it the premise of most salinity and aggregate dissolved solids estimations, conductivity is an early marker of variations in water quality condition. It likewise serves as a method for identifying contamination as specific toxins which do not break down into ions like unrefined petroleum can prompt lessened electrical conductivity. Similarly, it has been reported that higher conductivity readings during dry season and attributed them to evaporation while conducting a limnological study of the influence of domestic and agricultural effluents on three artificial lakes in Ibadan. The study also made the claim that dilution by rainwater may be the cause of conductivity readings falling during the wet season. According to the study, high conductivity values are a sign that the number of contaminating particles is growing. This is true as electrical conductivity values were highest in the dry season months and lowest in the months making up the rainy season.

Total Dissolved Solid (TDS) is a measurement of inorganic salts, organic matter and other dissolved materials in water. The concentration and composition of TDS in natural waters is determined by the geology of the drainage, atmospheric precipitation and the water balance (evaporation-precipitation). Total dissolved solids cause toxicity through increases in salinity, changes in the ionic composition of the water and toxicity of individual ions. Increases in salinity have been shown to cause shifts in biotic communities, limit biodiversity, exclude less-tolerant species and cause acute or chronic effects at specific life stages. In the present study, TDS ranged from 16.22±0.40mg/l (Oba River) to 19.03±0.31mg/l (Ogoloma River), which is lower than the 380.52-710.08mg/l.

The likely sources and causes of the major contributors to the elevated TDS concentrations are: increased alkalinity and calcium concentrations resulting from the weathering, efficient delivery, and accelerated dissolution of calcium carbonate found in the increased expanses of impervious surfaces and drainage systems in urbanized watersheds;  increased sulfate concentrations most likely resulting from more efficient delivery of deposited or spilled sulfur compounds from impervious areas and wastewater leaks or illicit discharges; and elevated chloride concentrations resulting from varied sources as wastewater leakage, runoff of lawn fertilizers, and spills or discharges of varied substances containing chlorides, including road salts

The mean turbidity values in the present study were between 2.98±0.20mg/l (Okrika River) to 8.10±0.93mg/l (Abuloma River). Turbidity estimations are frequently utilized as a marker of water quality considering clarity and known suspended solids in water [12]. Changes in turbidity between seasons could be connected to a few components like climate, especially overwhelming precipitation, which influences water stream, which thus influences turbidity. Precipitation can expand stream volume and thus stream, which can re-suspend settled dregs and breakup riverbanks [12].

Nitrate (NO3-) and nitrite (NO2-) are the main parts of the nitrogen cycle, which are naturally found in water. The presence of these inorganic nitrogen compounds is considered as an indicator of sewage water pollution. The mean nitrate concentration in the study ranged from 1.26±0.15mg/l (Georgetown River) to 3.25±0.63mg/l (Okrika River) which was higher than the 0.16±0.01mg/l to 1.145±0.045mg/l from a research report in River Galma, Zaria, Nigeria.  The difference in concentration could be due to environmental factors such as agricultural practices and pollution from waste generated by industries. Agricultural drainage, leachate from waste pile, and pollution from human and animal waste, are responsible for a vast increase of nitrate concentrations in surface water and groundwater. Freshwater fishes take up nitrite through gills, which leads to a high rate of accumulation. Sea-water fishes take nitrite through both gills and intestines (Jensen, 2003) [1]. Histopathological alteration of gills, esophagus, and brain was observed in acute exposure to sub-lethal concentrations of nitrates. Gill revealed lamellar fusion and hyperplasia of epithelium and lamellar shortening induced by necrosis. The esophagus showed hyperplasia of mucus cells and epithelium. The proliferation of glial cells and satellitosis (microglial cells sur-rounding neurons with swollen and prenecrotic neurons) were observed in the brain.

All the physico-chemical parameters examined recorded skewed correlationships with the abundance of the various fish species in the various rivers studied, having a positive relationship with the abundance of some fishes, and a negative relationship with the abundance of other fish species. These mixed outcomes could be due to varying physiologic responses the fishes expressed. Also, other factors that affect population growth such as availability of feeds, migration, disease outbreaks, exploitation, and species-specific fishing could have affected the overall fishes’ abundance.

4. Conclusion

The physico-chemical parameters recorded a skewed linear relationship with the abundance of the fish species in the study area. To maximize fisheries potential and preserve the best physiological condition of fish species, the physico-chemical parameters of surface waters at the landing sites should be continuously monitored to prevent further deterioration.

References

1. Adeogun, A. O. Impact of industrial effluents on water quality and fish fauna of the Elemi River, Nigeria. Journal of Environmental Biology, 2004; 25(3): 321-7.

2. Olaniyan, R. F. O. Influence of physico-chemical parameters on fish distribution in Lagos  Lagoon. Journal of Environmental Biology, 2010; 31(4): 567-72.

3. Parashor, R. S., Bisht, R. S., & Kumar, K. Effect of temperature on water quality parameters. Journal of Environmental Biology, 2007; 28(3): 567-72.

4. UNEP. Water quality for ecosystem and human health. United Nations Environment Programme, 2006.

5. Pandey, B. N., Kumar, V., & Tripathi, M. Impact of physico-chemical parameters on fish production. Journal of Environmental Biology, 2012; 33(4): 789-94.

6. Idowu, E. O., Adekunle, A. T., & Adebayo, O. T. Water quality and fish production in ponds. Journal of Environmental Biology, 2013; 34(4): 789-94.

7. Kamran, T., Khan, M. A., & Ali, S. Effect of water temperature on fish growth. Pakistan Journal of Zoology, 2013; 45(3): 789-94.

8. Premlata, V., & Vika, S. Dissolved oxygen and water quality. Journal of Environmental Biology, 2009; 30(2): 231-6.

9. Boyd, C. E. Water quality management for pond fish culture. Elsevier, 2004.

10. Abowei, J. F. N. The condition factor, length-weight relationship and abundance of Ilisha africana from Nkoro River, Niger Delta, Nigeria. Advance Journal of Food Science and Technology, 2010; 2(1): 6-11.

11. Chang, H. Spatial and temporal variations of water quality in the Han River and its tributaries, Seoul, Korea. Water, Air, and Soil Pollution, 2002; 141(1-4): 273-95.

12. EPA. Parameters of water quality: Interpretation and standards. Environmental Protection Agency, 2001.

13. WHO. Guidelines for drinking-water quality. World Health Organization, 2004.

14. Ogbeibu, A. E., & Victor, R. Hydrological studies of the River Ethiope, Nigeria. African Journal of Ecology, 1995; 33(3): 234-45.

15. Tassaduqe, S. H., Ali, M., & Salam, A. Effect of pH on fish production. Pakistan Journal of Biological Sciences, 2003; 6(12): 1023-7.

16. Walsh, C. J., Fletcher, T. D., & Ladson, A. R. Stormwater runoff and water quality. Australian Journal of Water Resources, 2002; 5(2): 123-30.

17. Chindah, A. C. The effect of salinity on periphyton community in a tropical estuary, Niger Delta. African Journal of Environmental Assessment and Management, 2004; 9: 45-56.

18. Nwankwoala, H. O., & Oborie, E. Impact of industrial activities on water quality in the Bonny River, Nigeria. Journal of Environmental Chemistry and Ecotoxicology, 2016; 8(4): 45-51.

19.  Fatoki, S. O., Muyima, N. Y. O., & Lujiza, N. Nutrient levels in surface waters of the Umtata River, South Africa. Water SA, 2001; 27(4): 467-74.

20. Batzler, W. E. Phosphorus and water quality. University of Wisconsin-Extension, 2002.

21. Davis, J. R., & Müller, R. Nutrient dynamics and agricultural runoff in estuarine systems. Environmental Monitoring and Assessment, 2021; 193(5): 245-9.

22. American Public Health Association. Standard methods for the examination of water and wastewater (23rd ed.). American Public Health Association, 2018.

23. USEPA. Water quality criteria for aquatic life. United States Environmental Protection Agency, 2006.