Review Article

IoT-Integrated Intelligent and Quality-Aware Solar Drying Systems: A Comprehensive Review of Technologies and Control Strategies

ABSTRACT

Solar drying has recently been recognized as a sustainable and energy-effective technique for minimizing post-harvest losses of agricultural produce, especially in regions with high solar intensity. Traditional open sun drying techniques, although cheap, are very sensitive to environmental changes which affect both the drying process and the resulting product quality. Advanced solar drying systems now include hybrid and mixed-mode systems which use direct and indirect systems to enhance thermal efficiency and drying rates. The Internet of Things (IoT) technologies now enable real-time monitoring and control for optimizing drying processes through their recent development. The review examines solar drying system technological development through the investigation of heat and mass transfer phenomena and different dryer designs and technical evaluations. The study focuses on IoT-based designs which use sensors and microcontrollers and Wi-Fi and GSM and LoRa and Bluetooth communication protocols and cloud computing and actuator-based automation. The paper contrasts conventional control methods like on-off and PID control with intelligent control methods like fuzzy logic control (FLC) and artificial neural networks (ANNs) and machine learning-based predictive models. The research demonstrates that intelligent control methods enable users to handle nonlinear drying kinetics together with solar irradiance fluctuations and drying processes which depend on quality factors. In general, intelligent solar drying systems with IoT can have a substantial potential to enhance drying homogeneity, lessen energy usage, decrease after harvest losses and improve food quality and safety. These innovations help to achieve sustainable agriculture and resilient food systems within the framework of climate variability and food security of the world.

Keyword: Solar drying, IoT integration, Intelligent control, Quality-aware drying, Machine learning, Hybrid solar dryer, post-harvest management, Sustainable food systems.

1.INTRODUCTION

Agriculture has always been the main source of food for the ever-increasing global population. With the steady increase in the global population and the continued existence of malnutrition in many countries, it is evident that food needs to be produced to a level where the losses during harvesting can be minimized. A large percentage of food loss is mainly due to microbial decomposition, which is greatly affected by moisture content.

Out of the various methods of preserving food, drying is probably the most dependable and the one that has been most extensively used to lower post, harvest losses. It essentially stops the proliferation of microorganisms and the enzymatic activity by lowering the moisture content to a level at which the food products can be safely preserved for a longer time. Thus, both pre, processing and post, processing operations are considered a part of the drying chain. They play a significant role in achieving the desired results. Specific preprocessing operations are dictated by the nature of the commodity; still, the proper execution of them can significantly contribute to drying efficiency and the long, term preservation of the product (Jebaraj & Khan, 2025)

Proper drying conditions must be maintained if the product is to retain the necessary nutrients, color, texture, and flavor. Otherwise, the end products may be discolored, the flavor may change, the texture may become different, the products' marketability may be reduced, and the products may be rejected after analysis. (Adeyeye et al., 2025) Majorly, the open sun drying method has been widely used due to its ease of operation and low labor cost. Dryers are at risk of several external factors such as rainfall, pest infestation, dust contamination, and microorganisms. These factors greatly influence the post, harvest losses of perishable food products, especially in low, and middle, income countries. (Ekechukwu & Norton, 1999; Ekechukwu et al., 2026). Moreover, open sun drying is very much dependent on weather conditions. Consequently, the drying period will be prolonged. A sudden change in the dryness of the atmosphere or the intensity of solar rays will disrupt the drying process, thus resulting in poor product quality.

1.1 Principles of Solar Drying

Pittia et al. (2016) described drying as a preservation method primarily aimed at reducing water activity to levels that inhibit microbial growth and delay quality deterioration. In the context of solar drying, the authors emphasized that moisture reduction plays a central role in extending shelf life while maintaining the structural and sensory properties of food materials. (Hodali & Bougard, 2001) explained that solar dryer’s function by capturing solar radiation through an absorber surface and converting it into thermal energy, which is subsequently transferred to the drying air. This heated air then passes over the product, enhancing moisture removal under controlled conditions. The authors highlighted that such systems offer a more reliable alternative to traditional open sun drying, particularly in terms of protection from environmental contamination and improved thermal performance. (Hodali & Bougard, 2001) also pointed out that most solar drying applications are designed as low-temperature systems to prevent thermal degradation of biological materials. Excessive temperatures, according to their findings, can adversely affect color, texture, and nutritional attributes, making temperature regulation essential in solar drying operations. Dincer and Cengel (2001) discussed the thermodynamic principles governing drying processes, noting that increasing the temperature of the drying medium raises the vapor pressure of moisture within the material. This vapor pressure gradient promotes internal moisture migration toward the surface, thereby accelerating evaporation. Further expanding on drying thermodynamics, Dincer and Sahin (2004) demonstrated that lowering the relative humidity of the drying air increases its moisture-carrying capacity and reduces the equilibrium moisture content of the product. Their thermodynamic model emphasized the importance of maintaining controlled air temperature and humidity to optimize drying efficiency.

Collectively, these studies indicate that solar drying is not merely a passive exposure to sunlight, but a controlled thermal process grounded in heat and mass transfer principles, aimed at improving product quality, drying efficiency, and post-harvest preservation performance.

1.2 Heat and Mass Transfer Mechanisms

Solar drying is governed by the simultaneous interaction of heat and mass transfer processes. Incident solar radiation is first captured and converted into thermal energy, which is transferred to the drying material through radiation, conduction, convection, or a combination of these mechanisms. The absorbed thermal energy increases the temperature of the product, creating a vapor pressure gradient that drives internal moisture migration toward the surface. This moisture then evaporates and is transported into the surrounding environment as water vapor. The drying process generally proceeds through two distinct stages: the constant rate period, where surface moisture evaporation dominates and external conditions such as air temperature, relative humidity, airflow velocity, and exposed surface area control the rate; and the falling rate period, where internal diffusion resistance within the material becomes the limiting factor. Accurate prediction of drying behavior therefore requires models capable of capturing the coupled heat and mass transfer phenomena, although many empirical and semi-theoretical models primarily focus on moisture loss characteristics while simplifying heat transfer mechanisms (Tiwari et al., 2016; Simo-Tagne et al., 2020).

Solar dryers are commonly classified based on their mode of heat transfer into direct, indirect, and mixed-mode systems. In direct-mode dryers, the product is directly exposed to solar radiation through a transparent cover, resulting in efficient energy absorption but limited temperature control. In indirect-mode dryers, solar radiation is absorbed by a separate collector, and the heated air is conveyed into an enclosed drying chamber, providing better regulation of drying conditions, particularly for heat-sensitive products. Mixed-mode dryers combine both mechanisms, allowing simultaneous direct radiation and convective heating, and may incorporate phase change materials (PCMs) to enhance thermal storage and stability. Depending on airflow mechanisms, solar dryers may also operate under natural convection, driven by buoyancy forces, or forced convection, supported by mechanical fans to improve drying uniformity and rate (Hodali & Bougard, 2001; Simo-Tagne et al., 2020).

Compared with traditional open sun drying, solar drying systems provide a more controlled thermal environment. In open sun drying, the surrounding air remains at ambient temperature while the product temperature rises due to direct solar absorption, leading to prolonged drying times and increased exposure to contamination, insects, birds, and unexpected weather variations. Such conditions increase the risk of discoloration, mould formation, and quality deterioration. In contrast, solar dryers elevate the drying air temperature above ambient levels, reduce drying time, protect the product within an enclosed structure, and improve final product quality attributes such as moisture content, color, taste, shape, and overall appearance (Ekechukwu & Norton, 1999; Hodali & Bougard, 2001).

1.3 Moisture Migration and Evaporation Mechanism

During the drying process, heat is required to evaporate moisture from the material, while airflow assists in carrying away the evaporated moisture from the product surface. Drying fundamentally involves two simultaneous mechanisms: (i) migration of moisture from the interior of the material to its surface, and (ii) evaporation of moisture from the surface into the surrounding air. A portion of the absorbed thermal energy penetrates the interior of the product through conduction, raising its internal temperature and generating water vapor. This vapor then diffuses toward the surface and is released into the surrounding environment, resulting in progressive dehydration of the crop. Moisture removal is typically rapid during the initial stages due to the presence of free moisture at the surface. However, drying behavior varies depending on the nature of the product and the rate at which internal moisture migrates to the surface (Sahu et al., 2016). The moisture removal rate is influenced by the material type, the presence of bound and unbound moisture, hygroscopic characteristics, and the thermophysical properties of the drying air. Hygroscopic materials tend to retain residual moisture due to internal binding forces, whereas non-hygroscopic materials can be dried to nearly zero moisture content. Drying proceeds at a constant rate until the critical moisture content is reached, beyond which the drying rate decreases. While both hygroscopic and non-hygroscopic materials exhibit a similar constant-rate period, differences arise in the falling-rate period: non-hygroscopic materials continue drying until moisture approaches zero, whereas hygroscopic materials experience a marked reduction in drying rate once unbound moisture has been removed (Aravindan et al., 2017). Overall, drying is a complex heat and mass transfer process governed by both external and internal variables. External factors include air temperature, relative humidity, and airflow velocity, while internal factors depend on surface characteristics, chemical composition (e.g., sugars and starches), physical structure (porosity and density), and product size and geometry. The primary objective of a drying system is to supply air at temperatures above ambient conditions, thereby increasing vapor pressure gradients and enhancing moisture removal efficiency (Akter et al., 2022)

Fig. 1. Schematic representation of coupled heat and mass transfer during solar drying

2. TYPES OF SOLAR DRYERS

Solar energy drying systems are classified mainly according to the mode of heating and the way that solar heat is utilized. Essentially, they can be split into direct (integral) and indirect (distributed) solar dryers, depending on whether the product is exposed to solar radiation or the heated air comes from the separated solar collector (Ekechukwu & Norton, 1999). The attempt to maximize and optimize solar energy while increasing energy efficiency in the drying process, resulted in four major types of solar dryers: direct, indirect, mixed, mode, and hybrid systems (Fudholi et al., 2010). Drying of agricultural products is one of the most promising areas of utilizing solar energy in an efficient and cost-effective way, especially in tropical and subtropical regions where solar radiation is very high (Ekechukwu & Norton, 1999; Fudholi et al., 2010). In addition to classification of solar dryers by heating technique, it is also possible to classify them by the mechanism of airflow during the drying process into active dryers and passive dryers. Active solar dryers, also known as forced convection solar dryers, use electrical power supplied by electric current generated by solar cells or the electrical power grid to fans or blowers to supply the heated air to the solar collector/dryer. Passive solar dryers, on the contrary, utilize the natural convection mechanism in which the convective flows are driven by buoyancy caused by the temperature differences in the system (Tiwari, 2016; Sodha et al., 1985). The energy requirement in the drying of agricultural products is mainly governed by the initial and final moisture content in the products as well as the properties associated with the thermodynamics of the process.

2.1 Direct Type Solar Dryer

The most basic and conventional type of solar drying technology is the direct type of solar dryer. In the direct type of solar dryer, the solar irradiation is directly exposed to the agricultural products through an enclosed space made of a transparent cover material (Prakash & Kumar, 2014). The transparent cover material allows the penetration of short-wave solar irradiation of wavelength 0.3-2.5 μm and blocks the re-emission of long-wave irradiation, resulting in the greenhouse effect that increases the temperature of air and products for moisture evaporation (Fudholi et al., 2010).

Direct solar dryers are commonly constructed as box-type or cabinet dryers. Typical components include:

- Transparent glazing cover (glass or polyethylene sheet)

- Darkened absorber base to enhance solar heat absorption

- Insulated side walls to minimize thermal losses

- Mesh trays for product loading

- Top and bottom ventilation openings to promote natural convection airflow

(Dare-Adeniran & Areola, 2022; Thakuria, 2018; Pakhare et al., 2016)

The working principle is based on the direct absorption of solar irradiance by the product surface, typically ranging between 500 and 1000 W/m² in tropical regions. Dark-colored crops exhibit high absorptivity (α ≈ 0.9), enabling efficient conversion of solar radiation into thermal energy (Kumar et al., 2021). The absorbed heat creates temperature gradients within the drying chamber, resulting in simultaneous heat and mass transfer. Peak chamber temperatures generally range from 45 to 70°C, depending on ambient conditions and glazing inclination, which is typically optimized between 10° and 30°C based on local latitude (Selvi et al., 2017; Ennissiou et al., 2023; Oubella et al., 2022). Airflow in direct dryers is mainly driven by buoyancy forces (natural convection), with air velocities typically ranging between 0.5 and 1.5 m/s. Moisture-laden air exits through chimney vents due to the thermosiphon effect.

2.1.1 Performance Characteristics

Empirical investigations report:

- Thermal efficiency: 10–25%

- Drying time reduction: 50–70% faster than open sun drying

- Suitable for thin-layer products such as herbs, spices, and small fruits

(Fudholi et al., 2010)

Table 1. Comparative Performance of Direct Solar Dryer Prototypes

| Study | Capacity (kg) | Peak Temp. (°C) | Efficiency (%) | Drying Time Reduction vs. Sun (%) |
| --- | --- | --- | --- | --- |
| Sharma (1990), Box dryer for fruits | 10 | 65 | 18 | 60 |
| Pruthvi Raj et al., (2025) Cabinet for corn | 15 | 70 | 22 | 70 |
| Prakash & Kumar (2014) | 5–20 | 45–70 | 10–25 | 50–70 |

Fig .2. Schematic diagram of a direct-type solar dryer illustrating greenhouse effect and natural convection airflow.

2.2 Indirect Type Solar Dryer

An indirect-type solar dryer is a system in which agricultural products are dried using heated air generated in a separate solar collector, thereby preventing direct exposure of the product to solar radiation (Ekechukwu & Norton, 1999). In this configuration, incident solar radiation is absorbed by the collector surface and converted into thermal energy. The generated heat is transferred to the drying air before it enters the drying chamber (Esper & Mühlbauer, 1998). The heated air then flows through the drying chamber by convection, removing moisture from the product while shielding it from direct sunlight. This arrangement helps reduce discoloration, nutrient degradation, and quality deterioration during drying (Bala & Woods, 1994).

A typical indirect solar dryer consists of a flat-plate solar air collector equipped with single or double glazing, a blackened absorber plate, and an inclination angle approximately equal to the local latitude (±15°) to maximize solar energy capture (Yassen et al., 2021; Tedesco et al., 2019). The collector is connected through insulated ducts to an enclosed drying chamber constructed from wood or metal. The drying cabinet generally contains multiple mesh or perforated trays, ventilation ports, and chimney outlets to facilitate airflow and moisture removal (Amedorme et al., 2013; Ennissioui et al., 2023; Zeeshan et al., 2024).

Although indirect solar dryers involve higher construction costs and greater structural complexity compared to direct solar dryers, they offer improved control over drying temperature and airflow. This enhanced control results in more uniform drying and better product quality (Prakash & Kumar, 2014). However, the higher initial investment—typically ranging from USD 100 to USD 500—may limit adoption among smallholder farmers. Furthermore, system efficiency can decline over time due to heat losses in air ducts (approximately 5–15%) and reduced collector performance caused by dust accumulation if proper maintenance is not ensured (Kumar et al., 2021).

Fig .3. Schematic diagram of an indirect-type solar dryer showing separate solar collector and drying chamber

Table 2. Comparative Performance of Indirect Solar Dryer Prototypes

| Study/Reference | Dryer Capacity (kg) | Peak Drying Air Temperature (°C) | Thermal Efficiency (%) | Drying Time Reduction vs. Open Sun (%) |
| --- | --- | --- | --- | --- |
| Behera et al. (2024) | 5–10 | 58 | 21.23 (system) | 43 (4 vs 7 days, mint leaves) |
| Afzal et al. (2023) | ~5 | 55–60 | 20–22 | 40–50 |
| UMPir equivalent (2023) | 10 | 65 | 23.4 (collector) | 50 |
| Kong et al. (2024) | 20–100 | 50–70 | 20–40 | 40–60 |

2.3 Hybrid Solar Dryer

Hybrid solar dryers, also referred to as mixed-mode solar dryers, combine both direct and indirect solar heating mechanisms within a single integrated system to achieve continuous and controlled drying. These systems may also incorporate auxiliary energy sources to enhance performance under fluctuating weather conditions. In hybrid configurations, agricultural products receive heat from two sources: direct solar radiation entering through a transparent cover and preheated air supplied by a separate solar air collector. (Rawat & Kumar, 2022; Kong et al., 2024; Afzal et al., 2023) This dual heating approach improves thermal utilization and enhances overall drying efficiency (Ekechukwu & Norton, 1999; Sharma et al., 2009; Kumar et al., 2021; Kong et al., 2024). A typical hybrid solar dryer consists of a flat-plate solar air heater (SAH) that preheats ambient air before directing it into the drying chamber. Simultaneously, a transparent roof allows shortwave solar radiation to directly heat the product trays inside the chamber. Natural convection is often promoted through chimney vents, while photovoltaic (PV)-powered fans may be used to provide forced convection and ensure uniform airflow distribution throughout the drying chamber (Afzal et al., 2023; Rawat & Kumar, 2022; Girhe & Mane, 2021). The combined effect of direct solar radiation and convective hot-air heating increases the drying chamber temperature by approximately 10–20 °C compared to purely indirect solar dryers. As a result, hybrid systems typically achieve thermal efficiencies ranging from 25% to 45% under peak solar conditions (Srimanickam et al., 2025; Rawat & Kumar, 2022; Girhe & Mane, 2021).

Fig .4. Schematic diagram of a hybrid (mixed mode) solar dryer showing combined direct solar radiation heating and indirect preheated air circulation.

Table 3. Hybrid Solar Dryer Performance Metrics

| Type | Capacity (kg) | Peak Temp. (°C) | Efficiency (%) | Backup System | Reference |
| --- | --- | --- | --- | --- | --- |
| Solar + Electric | 15-50 | 65-75 | 25-35 | Resistive heater | Paes et al. (2022) |
| Solar + Biomass | 20-100 | 60-70 | 20-30 | Combustion chamber | Rudra Solar Systems |
| Solar + PCM | 10-25 | 55-65 (stable) | 28-40 | Phase change mat. | Venkatesh et al. (2025) |
| Solar + PV fan/heater | 5-20 | 50-70 | 22-32 | Battery backup | Hasan et al. (2025) |

3. INTERNET OF THINGS (IOT) IN SOLAR DRYING SYSTEM

The Internet of Things (IoT) in agri-food systems refers to the integration of physical devices, sensing units, communication networks, control components, and cloud-based analytics platforms to enable real-time monitoring, automation, and optimization of agricultural production and food processing operations (Atzori et al., 2010; Wolfert et al., 2017).

At the foundational layer, sensors are deployed to collect real-time data on crop health, environmental conditions, and product quality. These include soil moisture sensors, temperature and humidity sensors such as DHT22 and capacitive sensors, multispectral cameras for NDVI-based crop health monitoring, and quality-related devices such as load cells and moisture meters. Modern sensor systems offer high measurement accuracy, typically around ±0.5% RH and ±0.1 °C, thereby supporting precise decision-making in agri-food operations (Ruiz-Garcia et al., 2009; Verdouw et al., 2016; Mansoor et al., 2025; Li et al., 2014).

Sensor data are transmitted through wired or wireless communication technologies such as Wi-Fi, GSM, LoRa, ZigBee, and MQTT-based protocols to local gateways or remote cloud servers for processing and storage. These communication systems enable reliable data transfer across farms and processing facilities, facilitating geographically distributed monitoring and control (Abid et al., 2025; Elijah et al., 2018; Miano et al., 2023).

Controllers, including microcontrollers and embedded processors such as Arduino, ESP32, NodeMCU, and Raspberry Pi, analyze sensor data and execute control algorithms based on predefined thresholds, proportional-integral-derivative (PID) strategies, or machine learning models. Increasingly, edge computing functions are incorporated into these systems to allow low-latency decision-making. (Arifin et al., 2024; Elwakeel et al.,2026; Dianda et al., 2025) For example, when temperature exceeds a specified threshold, cooling fans or ventilation systems can be activated immediately before transmitting data to higher-level platforms (Gubbi et al., 2013; Xu et al., 2022; Elwakeel et al., 2026).

Based on controller outputs, actuators regulate system operations. These may include solenoid valves for irrigation control, servo motors for vent and damper adjustment, DC fans for airflow regulation, pumps for fluid transfer, and relays for activating heaters or phase change material (PCM) systems. Such actuator-based automation enables rapid system responses—often within seconds—to maintain desired environmental and process conditions (Vermesan et al., 2014; Navarro et al., 2020; Mansoor et al., 2025).

Cloud platforms such as AWS IoT, ThingSpeak, and Blynk form the backbone of IoT-enabled agri-food applications by providing scalable data storage, real-time visualization dashboards, remote monitoring through web or mobile interfaces, and advanced analytics capabilities. Cloud-based machine learning algorithms support predictive analytics, decision support systems, supply chain traceability, and optimization of drying and storage processes (Botta et al., 2016; Wolfert et al., 2017; Mansoor et al., 2025).

The integration of sensors, controllers, actuators, and cloud computing enables closed-loop control systems in which real-time data measurements trigger automated corrective actions. In agri-food drying and post-harvest applications, such systems have been reported to reduce energy consumption by 20–30% and post-harvest losses by 15–25% (Verdouw et al., 2016; Elijah et al., 2018; Navarro et al., 2020). Overall, IoT-enabled agri-food systems enhance resource efficiency, transparency, traceability, product quality, and informed decision-making, thereby contributing to intelligent and sustainable agricultural production and food processing (Wolfert et al., 2017; Kamilaris & Prenafeta-Boldú, 2018).

3.1 Sensors Used in IoT-Integrated Solar Dryers

3.1.1 Temperature Sensors

Temperature sensors are essential for monitoring the drying air temperature in the solar air collector, drying chamber, absorber plate, product surface, and surrounding environment (Wanjale et al., 2025; Correa-Hernando et al., 2010). Commonly used devices include Type-K thermocouples (±0.5 °C), resistance temperature detectors (RTDs) such as PT100 (±0.1 °C), and digital sensors like DHT22/AM2302 (±0.5 °C) (TNAU, n.d.; Hasan et al.,2025; Correa-Hernando et al.,2010). Maintaining the drying temperature within an optimal range, typically 40–65 °C, is critical to prevent overheating, case hardening, and degradation of heat-sensitive nutrients (Al Maiman et al., 2021; Tomar et al., 2017; Wang et al.,2022)

3.1.2 Relative Humidity Sensors

Relative humidity (RH) sensors are used to estimate equilibrium moisture content (EMC), evaporation rate, and the moisture-carrying capacity of drying air (Correa-Hernando et al., 2010). Capacitive RH sensors such as DHT22 and HIH6130 (±2% RH accuracy) are commonly installed at inlet and outlet points of the drying chamber (Alkahdery et al., 2023). Laboratory-scale experiments may also use psychrometers. Maintaining low RH levels, typically below 30%, enhances moisture removal efficiency (Sanorchit, 2021; Correa-Hernando et al., 2010).

3.1.3 Air Velocity Sensors

Air velocity sensors measure airflow through the drying chamber, chimney, and solar air collector, directly influencing convective heat and mass transfer coefficients. (Venkatesh et al., 2025; Reyer et al., 2020). Air velocities between 0.5 and 3 m/s under natural or forced convection are commonly monitored using hot-wire anemometers (±0.05 m/s), vane anemometers, or differential pressure-based sensors. Optimized airflow ensures uniform drying, prevents moisture accumulation, and reduces drying time. (Dumbravă et al., 2024; Li et al., 2024; Adhikari et al., 2025).

3.1.4 Moisture Content Sensors

Moisture content sensors enable real-time monitoring of product moisture levels and accurate determination of drying endpoints. IoT-based solar dryers frequently use gravimetric analysis with load cells such as HX711 (±0.01 g accuracy), where moisture content is inferred from weight loss over time. Capacitive and resistance-based sensors are also used to measure changes in dielectric or electrical properties associated with moisture variation. Final product moisture is typically maintained within 10–15% to ensure safe storage and prevent over-drying. (Correa-Hernando et al., 2010; Yang et al., 2024)

3.1.5 Solar Radiation Sensors

Solar radiation sensors are essential for evaluating dryer performance and conducting energy balance analyses. Pyranometers and silicon-cell irradiance sensors (e.g., CM11K, ±5 W/ m2 accuracy) are typically mounted on the collector surface to measure global horizontal irradiance, generally ranging from 500 to 1000 W/m². Solar radiation data are used to correlate collector efficiency with insolation levels and to adjust control strategies according to weather variations (Kobusingye et al., 2025; Pruthvi Raj et al., 2025). In modern IoT-integrated solar dryers, sensor fusion using microcontrollers such as ESP32 or Arduino enables real-time processing, PID control, and cloud-based data logging through platforms like Blynk or Thing Speak. These systems have demonstrated temperature stability within ±2 °C, improved drying uniformity, and enhanced energy efficiency (Hasan et al., 2025; Mansoor et al., 2025).

Fig.5. Schematic diagram IOT Architecture of Solar Dryer

Table 4. IoT Sensor Specifications for Solar Dryers

| Parameter | Sensor Type | Accuracy | Typical Placement |
| --- | --- | --- | --- |
| Temperature | DHT22, PT100, Type-K thermocouple | ±0.1–0.5 °C | Chamber, absorber plate |
| Relative humidity | DHT22, HIH6130 | ±2 % RH | Inlet/outlet air |
| Air velocity | Hot-wire anemometer | ±0.05 m/s | Ducts, chimney |
| Moisture content | Load cell, capacitive sensor | ±0.01 g | Drying trays |
| Solar radiation | Pyranometer | ±5 W/m2 | Collector surface |

4. COMMUNICATION TECHNOLOGIES IN IOT-BASED SOLAR DRYERS

4.1 Communication Technologies in IoT-Integrated Solar Dryers

Communication technology forms the backbone of smart solar drying systems, serving as the bridge between sensors, controllers, cloud platforms, and end users. Through reliable communication links, real-time monitoring of drying parameters, remote adjustment of setpoints, performance analysis, and system notifications can be achieved. This connectivity reduces the need for continuous manual supervision and ensures that agricultural produce is safely managed throughout the drying process. The selection of an appropriate communication protocol depends on several factors, including transmission range, power consumption, data rate, availability of network infrastructure, and overall system cost (Osanaiye et al., 2017; Pruthvi Raj et al., 2025; Patil et al., 2024).

4.2 Wi-Fi Communication

Wi-Fi-based communication is commonly implemented in solar dryers located in areas with stable internet connectivity, such as research institutions, urban farms, and agro-processing facilities. Wi-Fi modules operating at 2.4 GHz, such as ESP8266, ESP32, and NodeMCU, typically provide communication ranges of 50–100 m and support high data transmission rates. These features enable real-time transmission of sensor data—including temperature, relative humidity, and moisture content—to cloud platforms such as Blynk and ThingSpeak. Consequently, users can continuously monitor drying parameters via dashboards on smartphones or computers (Wanjale et al., 2025; Ghafar et al., 2025; Paes et al., 2022). However, Wi-Fi systems generally consume higher power and offer limited coverage compared to long-range technologies, which restricts their suitability for remote or off-grid solar drying applications.

4.3 GSM Communication

For solar dryers installed in rural or isolated regions without Wi-Fi infrastructure, GSM communication provides long-range connectivity through cellular networks. GSM modules such as SIM900 and SIM800L support data transmission via SMS or GPRS services, allowing users to receive alerts regarding overheating, drying completion, or system malfunctions directly on their mobile devices. Although GSM systems involve network subscription costs and moderate power consumption, they are particularly suitable for off-grid monitoring because they operate independently of local internet infrastructure (Aldhaheri et al., 2024; Kerem & Yazgan, 2022).

4.4 LoRa Communication

LoRa (Long Range) communication technology has gained attention in decentralized solar drying systems due to its ultra-low power consumption and extended communication range, typically between 1 and 15 km. LoRa modules such as SX1278 and RA-02 operate in unlicensed sub-GHz frequency bands and are designed for low data rate transmission of essential parameters such as temperature and relative humidity. This makes LoRa particularly suitable for multi-node solar drying systems powered by batteries or photovoltaic panels, especially in remote agricultural areas (Aldhaheri et al., 2024; Ghafar et al., 2025).

4.5 Bluetooth Communication

Bluetooth technology, particularly Bluetooth Low Energy (BLE), is widely used in small-scale or laboratory-level solar dryer applications for short-range communication. Modules such as HC-05, HC-06, and BLE 5.0 are typically paired with smartphones or tablets for system configuration, calibration, and localized data acquisition. While Bluetooth offers low power consumption and fast response times, its limited communication range (generally 10–30 m) restricts its application to on-site operations rather than remote monitoring (Paes et al., 2022; Kishore et al., 2025)

Fig.6. Communication technologies used in Solar dryer technologies

5. QUALITY-AWARE DRYING CONTROL

Quality-aware drying control considers product-quality goal in the control logic for drying processes so that product moisture removal is achieved without deterioration of the product sensory, nutritional and safety properties (Xu et al., 2025). This is particularly critical for thermally and structurally sensitive agricultural products as suboptimal drying conditions may lead to adverse colour changes, depletion of bioactive compounds, texture damage and poor rehydration behavior (Ratti, 2001; Mujumdar, 2014). Quality-aware drying is a control principle in which operational parameters (air temperature, relative humidity, airflow rate and drying time) are adjusted in a way that is able to not only meet the target moisture content and storage stability but also key quality attributes such as colour, nutrient, texture, aroma, and microorganism safety (Chen & Mujumdar, 2008). According to Prakash & Kumar, (2014) and Simo-Tagne et al, (2020), there is need of such an approach because (1) many food and agricultural products are heat-sensitive and react non-linearly to drying stresses and (2) the solar drying ambience is inherently variable (solar irradiance, ambient humidity, wind). Research and reviews confirm that failure to consider quality can lead to poor end-products, even when moisture targets are achieved (Ratti, 2001).

6. DIFFERENCE BETWEEN CONVENTIONAL AND QUALITY-ORIENTED CONTROL.

Generally, conventional drying control strategies operate using fixed set-point temperatures or predefined time–temperature schedules, where feedback is often limited to air temperature alone. The primary objectives of such control approaches are to achieve faster drying or minimize energy consumption rather than to preserve product quality (Mujumdar, 2014). However, these conventional systems typically do not account for the dynamic state of the product, such as internal moisture gradients or temperature history. As a result, problems such as over-drying, case hardening, and non-uniform moisture distribution frequently occur, particularly during the falling-rate period of drying (Ratti, 2001). In contrast, quality-oriented drying control adopts a product-centric and multi-variable perspective. Instead of relying solely on-air temperature, this approach incorporates measurements of key process and product parameters, including moisture content, weight loss, surface temperature, color indices, and water activity. Drying conditions are then adjusted in response to these quality-related variables while still achieving the required final moisture level (Chen & Mujumdar, 2008; Shimpy et al., 2023). With the advancement of sensing technologies and digital systems, quality-oriented control can now be implemented through the integration of sensors, embedded processors, and cloud-based data analytics. (Pruthvi Raj et al.,2025) These technologies enable closed-loop control systems capable of adaptively modifying temperature, airflow rate, and auxiliary heating based on real-time quality indicators. Such systems not only help maintain nutritional, structural, and sensory quality but can also improve overall process efficiency by reducing unnecessary drying time and energy use (Kamilaris & Prenafeta-Boldú, 2018; Navarro et al., 2020).

6.1 Quality Parameters Affected During Drying

6.1.1 Nutritional Quality (Vitamins and Antioxidants

Drying is a process that destroys a significant percentage of heat, labile and oxidation labile nutrients such as vitamin C, vitamin A, carotene, phenolic compounds, and antioxidants. The nutrient content retained in the product is essentially the amount of nutrients that have not been degraded by the increased temperature and extended drying times during the drying process (Ratti, 2001; Mujumdar, 2014). It was also found that maintaining the drying temperature under control, such as drying at low temperatures, allowed more bioactive compounds to be retained (Chen & Mujumdar, 2008). It was reported that solar dryers with temperature control retained more antioxidants in the dried products (Prakash & Kumar, 2014).

6.1.2 Physical Quality (Color, Shrinkage, and Texture)

The drying conditions have a major impact on the physical properties such as texture, shrinkage, porosity, and color. A temperature increase or a rapid surface drying might lead to the formation of a hard crust, distortion, or shrinkage, which in turn may deteriorate the product's texture and the rehydration performance (Mujumdar, 2014). The disappearance of the product's color results mainly from enzymatic or non, enzymatic browning, pigment decomposition, or overexposure to solar irradiation (Ratti, 2001).The use of controlled solar drying systems has been found to result in better texture retention, less case hardening, and fewer structural damages in the dried product when compared to conventional sun drying methods (Shimpy et al., 2023).

6.1.3 Sensory Quality (Flavor and Aroma)

Flavors and aroma volatiles mentioned above are volatile in nature, highly susceptible to temperature, and therefore their quality can be altered as they may evaporate during the drying process leading to the loss of their sensory quality (Chen & Mujumdar, 2008). Temperatures and exposure times will typically worsen this quality loss, whereas a good combination of temperature and exposure time would result in better retention of the desirable sensory qualities of the produce, since solar dryers can provide this temperature optimization, as mentioned in Ratti (2001). This better retention of flavor and aroma was also pointed out in Prakash & Kumar (2014).

6.1.4 Microbial Safety

One of the important goals of drying is to enhance the microbial safety of products through decreasing moisture content and a value to such a level where microbial growth can be limited. When drying is done improperly, for instance, if the drying rate is slow and the ambient condition is humid, bacteria, yeasts, and molds may grow, thus resulting in spoilage and leading to health problems for consumers (Mujumdar, 2014).The right drying conditions can decrease water activity (aw) values below the critical microbial growth threshold of 0.6, preventing pathogen proliferation and extending product shelf life (Ratti, 2001). Solar dryer systems create hygienic drying environments through regulated temperature (40-60°C) and controlled airflow rates, significantly reducing contamination risks compared to open sun drying (Alp & Bulantekin, 2021).This controlled microenvironment achieves uniform moisture removal (target: 8-12% wb), inhibits mold growth, and eliminates aflatoxin formation while maintaining aw < 0.6 for long-term storage stability (Alp & Bulantekin, 2021).

7. CONTROL STRATEGIES FOR IOT-INTEGRATED SOLAR DRYERS

The efficiency of the solar dryers integrated with IoT mainly depends on how well the control strategy can regulate the temperature, relative humidity, and air flow considering changing environmental and solar power conditions. Since the power from solar sources varies periodically, if the control strategies are not good, it may result in temperature fluctuations and energy being wasted. As a result, both simple and complex control strategies were used to manage the temperature and improve the solar dryers' efficiency, and the traditional control strategy was favored because of its simplicity and the fact that it was effective.

7.1 Conventional Control Methods

7.1.1 On–Off Control

On, off control is the simplest form of feedback control used in solar drying systems, and it is also called bang, bang control or thermostat control. The on, off control method works by turning the actuators such as fans, heaters, or dampers, fully on or off, depending on the set, point values of the measured variable such as drying air temperature or relative humidity. A hysteresis band, e.g. 55±5°C, is normally used in the field to stop the unstable control actions from happening when the measured variable is fluctuating around the set, point value. Recent implementations of IoT projects involving solar dryers make mention of on-off control logic mechanisms involving microcontrollers such as Arduino and ESP32, where sensors are triggered to switch on fans/heaters once they cross specific control limits. (Pruthvi Raj et al., 2025). Elwakeel et al. (2023) demonstrates Arduino-based date fruit monitoring with PV integration, validating on-off control for premium crop quality preservation. However, the limitations of on-off controllers have been pointed out in some studies, particularly in drying processes, such as the temperature variation within a range of ±5 to 10°C, long settling times, typically 10 to 30 minutes, and large numbers of switchings applied to the actuators. (Nkolokosa, 2023). Such oscillations are considered as unwanted fluctuations, particularly at the time of the falling rate period of the drying process. It is very important to keep the conditions unchanged during the falling rate period for the continuous maintenance of the quality of the product. (Scale, Time, Temperature)

7.1.2 PID Control

The most frequently used classical control technique in IoT-enabled smart solar dryers is the PID controller, measuring error e(t) = Tsetpoint - Tmeasured. Typical tuning ranges include Kp = 1-10, Ti = 10-100s, Td = 1-10s using Ziegler-Nichols method (Melinda et al., 2023; Nkolokosa, 2023).

ut=Kpet+Ki0teτ dτ+Kdddtet

where e(t) is the temperature error (Melinda et al., 2023)

Recent experimental studies have revealed that temperature control regulation for PID, control of solar dryers takes its position favorably as opposed to traditional on/off control arrangements. The report indicates that some of the performance for this kind of system include an overshoot of less than 5%, settling times of less than 5 min, and steady states temperature errors of less than 1°C (Yang et al., 2023; Melinda et al., 2023). Aside from that, there have been instances where the PID was actually instrumental to the realization of the IoT platforms that utilize ESP32/STM32-based controllers towards the facilitation of real time control as well as monitoring. Their functions have been taken a step further in certain instances through the incorporation of so-called adaptive/intelligent tuning approaches that have employed fuzzy PID, network-based PID controllers, among others (Yang et al., 2023; Nkolokosa, 2023).

Although PID control has the above advantages, the tuning of the PID controller is critical, and the performance of the controller might be compromised at highly nonlinear conditions of drying operation or rapid variation of solar irradiance. PID control, however, remains the benchmark for quality-focused solar drying control as a basis of comparison to other advanced intelligent control methods.

Fig.7. Temperature vs. Time response curve

Table 5. Comparison of on–off and PID control strategies in solar dryers

| Control Method | Overshoot | Settling Time | Steady-State Error | Reference |
| --- | --- | --- | --- | --- |
| On–off | ±5–10 °C | 10–30 min | High oscillation | Pruthvi Raj et al., (2025) |
| PID | <5% | <5 min | <1 °C | Nkolokosa, D. (2023) |

7.2 Intelligent Control Approaches

Drying kinetics being nonlinear along with solar energy being intermittent, conventional control strategies is a major issue for solar dryers. Consequently, IoT, integrated solar dryers have started implementing intelligent control methods increasingly to address the dilemma. These techniques can effectively handle system nonlinearity and uncertainty as well as multiple, variable interactions, and at the same time, they facilitate quality, aware drying objectives.

7.2.1 Fuzzy Logic Control (FLC)

The use of fuzzy logic in the control of solar dryers is widespread because it does not rely on the mathematical simulation of the drying process. The fuzzy logic used in the control process involves several linguistic rules developed based on expert knowledge, for instance, “if the temperature is high and the relative humidity is low, then the heating should be reduced.” The most used fuzzy logic controllers use a combination of membership functions and Mamdani/Sugeno inferences, as they offer the capability to process various input variables, for example, the drying media’s temperature, moisture content, and relative humidity, and corresponding output variables, for instance, fan speed, damper setting, and/or additional heaters (Abakarov et al., 2012; Reyes et al., 2014).

Field-deployed fuzzy-controlled solar dryers demonstrate superior disturbance rejection compared to PID controllers, maintaining temperature overshoot below 3%, reducing settling times by 20%, and ensuring stable operation despite irradiance fluctuations and sensor noise—critical advantages for real-world agricultural applications (Reyes et al., 2014; Nkolokosa, 2023).

7.2.2 Artificial Neural Networks (ANNs)

Multilayer perceptron neural networks effectively capture nonlinear drying dynamics, predicting moisture content, drying rates, and temperature profiles from solar radiation, temperature, and humidity inputs. These data-driven models eliminate the need for complex heat/mass transfer equations while achieving high prediction accuracy for real-time IoT control applications (Yang et al., 2023; Nkolokosa, 2023). It has also been demonstrated that ANN models are capable of producing predictive results of high precision, as evidenced by the fact that the coefficient of determination (R²), which is obtained from the prediction results of ANN models, has achieved a value greater than 0.98, while the root mean square error (RMSE), which is another metric of the prediction effect of ANN models, is less than 1% at the same time (Bangar et al., 2023; Gupta et al., 2024; Tripathy & Kumar, 2009). In the context of the IoT, ANN models can be employed to implement the control systems of some devices as a means of pertinent feedforward control, thereby greatly improving the performance of the control systems of the devices, which could be either PID control systems or conventional control systems. In such cases, the error of the control systems is reduced to the extent of less than 1% after implementation (Liang et al., 2018; Buettner et al., 2022; Chen et al., 2019; Velichko et al., 2023).

7.2.3 Machine Learning–Based Prediction and Control

Besides the conventional neural networks, other advanced machine learning (ML) methods, including support vector regression (SVR), random forest (RF), long short-term memory (LSTM) networks, and reinforcement learning (RL), were also recently investigated for the control of solar dryers. They aim to utilize the available historical data during solar drying, real-time measurements, and weather forecasts to forecast the optimal temperature and humidity levels or to choose the control strategies adaptively. (Choubey et al., 2025; Cetina-Quiñones et al., 2023; Srimanickam et al., 2025)

However, latest study shows that machine learning prediction models can predict drying behavior and thermal performance with errors less than 1%, a very significant improvement compared to conventional regression and empirical models based on variable climatic conditions (Arunkumar et al., 2024; Srimanickam et al., 2025; Mule et al., 2024). Moreover, reinforcement learning techniques also enable this method to establish control policies that are efficient enough and maximize the process in terms of reward functions, for example, minimizing the energy required to maintain product quality. Predictive and adaptive control techniques have huge prospective in maintaining quality-sensitive solar drying products to avoid the adverse effects of overheating, over-drying, etc. (Li et al., 2025; Lee & Choi, 2019; Alghassab, 2025).

Fig.8. Intelligent Control Framework for IoT-Based Solar Dryer

Table 6. Comparison of conventional and intelligent control approaches in solar dryers

| Control Method | Nonlinearity Handling | Tuning Requirement | Typical Accuracy | Reference |
| --- | --- | --- | --- | --- |
| On-Off | Poor | None | ±5-10°C fluctuations | Pruthvi Raj et al. (2025) |
| PID | Moderate | Manual/Ziegler-Nichols | <5% overshoot, <1°C SSE | Melinda et al. (2023) |
| Fuzzy Logic | Excellent | Rule-based (expert) | RMSE 1-2% | Abakarov et al. (2023) |
| ANN | Excellent | Data-driven training | R² > 0.98, RMSE <1% | Arunkumar et al. (2024) |
| Advanced ML | Very High | Training-based | Error < 0.002% | Arunkumar et al. (2024) |

7.3 Decision-Based Drying Control

Decision, based drying control means a change in the way we understand drying from a traditional rule, based or a time, controlled operation to a decision, making process that is dynamic and adaptive, and driven by quality control as well. In IoT, enabled solar drying systems, it means always changing the best environment for drying and deciding the perfect point of finishing the drying of a particular material based on the quality requirements of the material being dried. (Mule et al., 2024; Kabeer et al., 2025; Rulazi et al., 2023). This approach, in this system of IoT-integrated solar dryers, makes use of dynamically variable parameters like sensor and predictive technology to recurrently change drying conditions and to arrive at the optimum endpoint of solar drying based on quality requirements. (Elwakeel et al., 2023; Nkolokosa, 2023)

By means of adaptive temperature and airflow control, methods change or vary the setpoints for drying air within certain limits specific for different crops, typically from 40°C to 65°C for temperature and from 0.5 to 3 m/s for airflow, depending on the immediate changes in solar irradiance, temperatures, and relative humidity, which are inherent in solar drying processes. (Elwakeel et al., 2023; Pruthvi Raj et al., 2025). Thus, adaptive controllers are capable of intentionally considering variations in solar irradiance, temperature, and relative humidity, unlike conventional fixed setpoint PID controllers.  (Elwakeel et al., 2023; Nkolokosa, 2023)

The latest IoT, enabled solar dryers to have been equipped with gain, scheduled PID control or MPC methods integrating short, term solar and ambient condition forecasts, which are generally delivered via weather APIs, for optimal control strategies that lead to the least use of auxiliary energy. (Sharma et al., 2025; Mohrir & Prashanth, 2025)

The real applications will integrate these prediction algorithms with rule, based logic architectures whereby phase change materials or supplementary heat generators will be activated once solar irradiance values drop to certain critical levels (e.g., < 400 W/m2) (Toshniwal & Karale, 2024; Venkatesh et al., 2025). The adaptive control of airflow enhances drying uniformity by increasing the airflow during the diffusion-controlled periods and decreasing it at high temperatures to avoid case hardening. (Kobusingye et al., 2025; Asrate et al., 2025)

7.3.1 Quality-Based Endpoint Detection

Basic ending points of drying only by time or final moisture content are replaced by quality, based endpoint detection that determines by various quality parameters the right moment to finish. Decision based solar drying finds the endpoint when the product quality standards set beforehand are achieved, thus it is safe as far as moisture is lowered and over, drying is prevented. IoT system consisting of integrated systems have used a variety of multi, methods such as gravimetric, moisture content, drying rate trend analysis, and sensing methods such as RGB, to carry out decision tree classification or even SVM classification for automatic termination of the drying process. (Elwakeel et al., 2023; Pruthvi Raj et al., 2025).

The drying requirements which normally are continuing and finalized by the authors have been mainly finalize with moisture content below 15%, constant moisture content within 2%, and a change in color, such that E < 20.Machine learning also helps with endpoint detection, thus these values being very accurately predicted to a level of 95%, which in turn results in shorter drying time as compared to sun drying. (Elwakeel et al., 2023; Pruthvi Raj et al., 2025)

7.3.2 Role in Quality-Aware Solar Drying

The combination of adaptive regulation and quality, based termination in decision, based drying control not only instantly aligns with the objectives of quality, aware solar drying, but also makes it possible to achieve an intelligent trade, off between efficiency, energy consumption, and quality, thus making decision, based drying control very compatible with the requirements of solar dryers with IoT connectivity under highly variable climatic conditions. (Pruthvi Raj et al., 2025; Kobusingye et al., 2025; Toshniwal & Karale, 2024).

Table 7. Examples of decision-based control strategies in IoT-integrated solar dryers

| Control Aspect | Method | Target Performance | Reference |
| --- | --- | --- | --- |
| Temperature & airflow | MPC / gain-scheduled PID | ±1.5 °C, 1–2 m s⁻¹ | Nkolokosa, D. (2023) |
| Endpoint detection | SVM + moisture & color | MC < 15%, ΔE < 20 | Liu et al. (2024) |

8. CHALLENGES AND LIMITATIONS

8.1 Sensor Accuracy and Calibration Issues

Effective quality, conscious drying control strategies largely depend on the precise measurements of temperatures, relative humidities, airflow rates, and moisture content, to name a few. Inexpensive sensors that are used in IoT devices may undergo measurement drift, have cross, sensitivity, and get damaged when exposed to extreme temperatures and relative humidities for long durations. Errors may be caused by poor calibration of these sensors, and such errors may result in inappropriate adjustments, in terms of quality and consistency of drying process outcomes. (Kobusingye et al., 2025; Miano et al., 2023; Ma'arij & Yudhana, 2023).

8.2 Internet Connectivity in Rural Areas

It is noted that many IoT, enabled solar dryers, such as IoT, based solar dryers, utilize wireless communication and "cloud platforms" to perform functions, e.g., "data storage, " "visualization, " and "prediction." However, the absence of good internet connectivity in remote areas is usually considered the primary problem for IoT, based approaches to agriculture by the relevant studies, which also agree that interruptions in connectivity have a negative impact on the system's responsiveness and continuity. (Pruthvi Raj et al., 2025; Liu et al., 2024).

8.3 Cost and Scalability

Even though the prices of IoT devices keep going down, the overall cost of an IoT system is still a problem, mainly because of its different components such as sensors, controllers, communication equipment, energy storage devices, and maintenance. Economically smallholder farmers cost mostly farmers in the developing economies, as we know from the reports, the small, scale models that are hard to be commercially viable while keeping the scalability solar dryers are one of the challenges in solar drying technology. (Pruthvi Raj et al., 2025; Liu et al., 2024).

8.4 Environmental Variability

On the other hand, solar drying systems may be affected by environmental changes such as variations in solar irradiation, temperature, humidity, and wind speeds. Thus, unstable conditions may arise in solar drying systems, even when the system is controlled by adaptive and intelligent control systems. Severe changes in weather conditions may lead to uneven drying and deteriorated product quality. (Kobusingye et al., 2025; Toshniwal & Karale, 2024)

8.5 Data Management and Reliability

Solar dryers based on IoT technology generate large volumes of diverse data from distributed sensing devices. To make decision control, it is extremely important to ensure reliable data transmission, storage, synchronization, and integrity. Loss of data information, a sensor getting out of order, and/or the delay in data transmission may result in a decreased system reliability and user trust. Moreover, the lack of data standard format and quality assessment schemes are a hindrance to the long, term performance evaluation of solar dryers (Botta et al., 2016; Verdouw et al., 2016).

9. FUTURE RESEARCH DIRECTIONS

9.1 AI-Driven Self-Learning Solar Dryers

Recent trends in the domain of smart drying and Industry 4.0 principals describe the use of reinforcement learning-based control policies like the DQN framework. This will allow the drying system to learn the optimal temperature and humidity control policies based on the analysis of the operational data and maximize the reward functions based on product quality and energy efficiency. Additionally, the multimodal sensor control will be used to control the drying system adaptively based on various crop types without any physical changes. Such drying systems have been reported to reduce energy consumption with the same product quality under varying climatic conditions (Kamilaris & Prenafeta-Boldú 2018; Liakos et al., 2018)

9.2 Digital Twin of Solar Dryers

Nevertheless, breakthroughs in agricultural digital twins have demonstrated the capability of PINNs to represent the complexities of the heat/mass transfer process, and at the same time, being able to stay coordinated with the IoT data sensors in real time. Digital twins technology, among others, is believed to be beneficial to smart agriculture for such applications as simulation, prediction of system failure, and evaluating "what, if" scenarios, say those who study this technology. As an example, digital twins for solar dryers could be used for predictive maintenance or for optimizing the physical dimensioning of the phase, change materials. (Tao et al., 2019; Fuller et al., 2020)

10. Techno-Economic and Life Cycle Perspectives

Solar drying systems offer farmers lower capital and operating costs compared to electric or open-sun methods, with IoT integration adding modest upfront expenses but boosting efficiency and ROI. (Kimaro et al., 2024)

10.1 Capital Cost Comparison

Solar dryers range from $100-500 for basic indirect models to $3,000-5,500 for hybrid or 1-tonne capacity systems, 45% cheaper than electric convective dryers ($0.86/kg operating). IoT adds $50-300 (sensors like DHT22, ESP32 modules, cloud setup), raising totals by 5-15% but enabling remote monitoring. (Akowuah et al., 2021; Rulazi et al., 2024; Mohrir & Prashanth, 2025)

Table 8: Capital Cost Comparison of Solar Dryers with IoT Integration

| Dryer Type | CapEx (USD) | vs. Electric | IoT Addition |
| --- | --- | --- | --- |
| Basic Indirect | 100-500 | -45% | +$50-100 |
| Hybrid Solar | 3,000-5,000 | -50%+ | +$200-300 |
| Solar-Biomass (1T) | 5,263 | N/A | +10% |

10.2 Energy Savings

Solar drying systems provide greater advantages than open air drying which is the conventional drying practice. The closed system of solar dryers together with their greenhouse effect. The closed system of solar dryers enables the drying process to produce temperatures which exceed 20, 50% of actual ambient temperature. In most cases whose details are available, the drying period has been found to be shortened by 40, 70% as compared with open, sun drying. The elements which enhance efficiency also function as dual elements which decrease PHL (post, harvest losses) because these elements make up 30 to 40% of total agricultural losses which developing countries experience through weathering and microbial activity and contamination. IoT technology development makes this system better because of its advanced capabilities. The system can automatically change to solar and environmental variations by using temperature and relative humidity data in real, time. Solar dryers which use IoT technology can achieve energy efficiency improvements of 20 to 30% because the system prevents overheating and over drying and stops workers from using unnecessary heating components. Improved product quality and drying consistency happen because the total energy usage decreases between 15 and 25%. The smart control system of this system ensures that energy consumption happens only when energy needs arise which leads to improved sustainability and cost-effectiveness for the drying operation. (Sharma et al., 2025; Suraparaju et al., 2024; Kimaro et al., 2024)

10.3 Payback Period

The typical payback period ranges from 1.1 to 2.7 years, with reported NPV values of approximately ₹4.0–29.3 lakh over 10–20 years and IRR between 38–84%. IoT integration can further reduce the payback to under two years by significantly increasing annual profits (≈₹13.0 lakh compared to ≈₹1.3 lakh for open-sun drying) (Suraparaju et al., 2024; Kimaro et al., 2024; Mohrir & Prashanth, 2025).Solar drying systems are cost, effective to small, scale farmers if the Benefit Cost Ratio (BCR) reaches about 1.48. This means that the financial benefits from the project surpass its costs. Community models also have a huge potential. Approximately, 210 units can generate employment for almost 420 rural youth. By reducing post, harvest losses, farmers would be able to save about 4.7 lakh, income leakage which is regularly faced by poor drying methods and open, sun drying. The government's subsidy and agricultural development programs, especially in the states like Madhya Pradesh and other parts of India, can also facilitate the promotion of solar drying systems as the requirement of the initial investment will become less. The initial fear of farmers can be due to the high cost of advanced systems or IoT, based systems, however, the long, term financial benefits justify the economic logic of the investment. For instance, the net gains in total value over a period of 15 years can be more than 22 lakh, which clearly shows that the financial gains are far more significant than the cost of investment. (Ndukwu et al., 2023; Rulazi et al., 2024)

11. CONCLUSION

This review critically assessed the innovation of solar drying technologies over a period, exposing the transition from the most traditional forms of direct or indirect solar drying to modern hybrid solar drying machines enhanced by Internet of Things (IoT) technologies. Although solar drying machines offer better protection and thus, speed up the drying process when compared to the conventional open, sun drying methods, these drying machines basically do not have real, time dynamic control over the key parameters like temperature, humidity, and airflow. New inventions resulted in an intelligent control system that integrates sensors, microcontroller technologies, the cloud, and data analytic techniques for the real, time monitoring of solar drying processes. The recent addition of communication technologies such as Wi, Fi, GSM, LoRa, and Bluetooth has further enhanced solar drying processes by providing for remote operation of the drying process. The present review mainly discusses the changes from time, or temperature, based drying to quality, aware drying control. The traditional methods of drying were based only on the reduction of moisture, whereas nowadays the decision is made based on the retention of nutrients, color stability, texture preservation, and safety from microorganisms in the methods of quality, aware control. Adaptive control methods, smart algorithms, and end-point detection techniques enable control of the end of the drying process based on the quality characteristics of the products, as opposed to a predetermined schedule. The benefits of such control include better product uniformity, reduced over, drying waste, and efficient energy use. Solar dryers using IoT technology could be one of the most hopeful and sustainable post, harvest management systems as they will utilize solar energy, which is a clean source, thus cutting down on fossil fuel usage. Besides, these drying systems can potentially reduce food wastage, create new food products and hence, ensure the sustainable and resilient food systems through the synergy of solar technology, based drying plus digital agriculture technologies. In addition, if the subsequent investigations are directed towards such areas as AI, based self, learning solar dryers, digital twin technology, blockchain, and protocol development, then IoT, integrated solar drying systems worldwide can be powerful levers towards the establishment of sustainable and resilient food systems.

Disclaimer (Artificial intelligence)

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

12. REFERENCE

- Abakarov, A., Jiménez-Ariza, H., Correa-Hernando, E., Diezma, B., Arranz, F. J., García-Hierro, J., Robla, J. I., & Barreiro, P. (2023). Control of a solar dryer through using a fuzzy logic and low-cost model-based sensor. Computers and Electronics in Agriculture, 205, Article 107604, 1-15.

- Abakarov, A., Sulaiman, S., & Aisyah, S. (2012). Fuzzy logic controller for solar dryer temperature regulation. International Journal of Renewable Energy Research, 2(3), 441–448.

- Abid, N. B., Khattab, A. M., Harb, H. A. M., & Souani, C. (2025). An IoT-based smart agriculture system using LoRa and cloud monitoring for automated greenhouse control. International Journal of Computer Applications, 187(52), 7–15

- Adeyeye, O. A. S., Ashaolu, J. T., & Babu, S. A. (2025). Food drying: A review. Agricultural Reviews, 46(1), 86–93.

- Adhikari, N., Garg, H., Davidsson, H., Baral, B., & Andersson, M. (2025). Flow uniformity inside the drying chamber of a heat exchanger-based solar dryer – numerical analysis with smoke flow visualization as experimental validation. Results in Engineering, 26, Article 105553, 1–15.

- Afzal, A., Iqbal, T., Ikram, K., Anjum, M. N., Umair, M., Azam, M., Akram, S., Hussain, F., Ameen ul Zaman, M., Ali, A., & Majeed, F. (2023). Development of a hybrid mixed-mode solar dryer for product drying. Heliyon, 9(3), Article e14144.1-15

- Akowuah, J. O., Bart-Plange, A., & Dzisi, K. A. (2021). Financial and economic analysis of a 1-tonne capacity mobile solar-biomass hybrid dryer for maize drying. International Journal of Agricultural Economics, 6(3), 103–112.

- Akter, F., Muhury, R., Sultana, A., & Deb, U. K. (2022). A comprehensive review of mathematical modeling for drying processes of fruits and vegetables. International Journal of Food Science, 2022, Article 6195257, 1–10

- Al Maiman, S. A., Albadr, N. A., Almusallam, I. A., Al-Saád, M. J., Alsuliam, S., Osman, M. A., & Hassan, A. B. (2021). The potential of exploiting economical solar dryer in food preservation: Storability, physicochemical properties, and antioxidant capacity of solar-dried tomato (Solanum lycopersicum) fruits. Foods, 10(4), Article 734, 1–13

- Aldhaheri, L., Alshehhi, N., Manzil, I. I. J., Khalil, R. A., Javaid, S., Saeed, N., & Alouini, M.-S. (2024). LoRa communication for Agriculture 4.0: Opportunities, challenges, and future directions.

- Alghassab, M. (2025). AI-driven solar air heating: Reinforcement learning optimization of thermoelectric systems for enhanced thermal-electrical performance. Case Studies in Thermal Engineering, 76, Article 107304, 1-20.

- Alkahdery, L. A., Yurchenko, A. V., Mohammed, J. A.-K., & Neshina, Y. G. (2023). Automated temperature and humidity control and monitoring system for improving the performance in drying system. Eurasian Physical Technical Journal, 20(2), 32–40.

- Alp, D., & Bulantekin, Ö. (2021). The microbiological quality of various foods dried by applying different drying methods: A review. European Food Research and Technology, 247(6), 1333–1343.

- Amedorme, S. K., Apodi, J., & Agbezudor, K. (2013). Design and construction of forced convection indirect solar dryer for drying Moringa leaves. Scholars Journal of Engineering and Technology, 1(3), 91–97.

- Aravindan, V., Dineshkumar, A., Giriprasath, B., Karthikeyan, V., & Ebenezer, D. (2017). Moisture removal rate of solar dryers – A review. Journal of Chemical and Pharmaceutical Sciences, Special Issue 7, 218–221.

- Arifin, M. A., Pangaribuan, P., Pramudita, B. A., & Megantoro, P. (2024). Controlling temperature using proportional integral and derivative control algorithm for hybrid forced convection solar dryer. International Journal of Electrical and Computer Engineering (IJECE), 14(1), 239–247

- Arunkumar, P. M., Balaji, N., Madhankumar, S., & Mohanraj, T. (2024). Prediction of red chilli drying performance in solar dryer with natural energy storage element using machine learning models. Journal of Energy Storage, 101(Part A), 1-18.

- Asrate, D. A., Tadle, F. B., & Mihretie, Y. D. (2025). Enhancement of airflow uniformity in multi-tray dryers through a CFD-integrated sequential linear programming framework. Scientific Reports, 16, Article 4226, 1-22.

- Atzori, L., Iera, A., & Morabito, G. (2010). The Internet of Things: A survey. Computer Networks, 54(15), 2787–2805.

- Bala, B. K., & Woods, J. L. (1994). Simulation of indirect natural convection solar drying of rough rice. Solar Energy, 53(3), 259–266.

- Bangar, A., Jadhav, V., Kashid, S., Waman, S., & Mule, R. (2023). AI-enhanced optimization of solar drying system for agricultural produce: A sustainable approach. International Journal of Creative Research Thoughts (IJCRT), 11(11), 184–192

- Behera, D. D., Mohanty, R. C., & Mohanty, A. M. (2024). Experimental investigation of a hybrid solar dryer for vegetable drying with and without phase change material. Journal of the Brazilian Society of Mechanical Sciences and Engineering, 46, Article 303, 1–12.

- Botta, A., De Donato, W., Persico, V., & Pescapé, A. (2016). Integration of cloud computing and Internet of Things: A survey. Future Generation Computer Systems, 56, 684–700.

- Buettner, M. A., Monzen, N., & Hackl, C. M. (2022). Artificial neural network based optimal feedforward torque control of interior permanent magnet synchronous machines: A feasibility study and comparison with the state-of-the-art. Energies, 15(5), Article 1838, 1–28

- Cetina-Quiñones, A. J., Santamaria-Bonfil, G., Medina-Esquivel, R. A., & Bassam, A. (2023). Techno-economic analysis of an indirect solar dryer with thermal energy storage: An approach with machine learning algorithms for decision making. Thermal Science and Engineering Progress, 45, Article 102131, 1-20.

- Chen, M., Challita, U., Saad, W., Yin, C., & Debbah, M. (2019). Artificial neural networks-based machine learning for wireless networks: A tutorial. IEEE Communications Surveys & Tutorials, 21(4), 3039–3071

- Chen, X. D., & Mujumdar, A. S. (2008). Drying technologies in food processing. Blackwell Publishing.

- Choubey, P., Sinhmar, H., & Tiwari, S. (2025). A review on machine learning assisted solar drying system with phase change material. Journal of Energy Storage, 119, Article 116403, 1-24.

- Correa-Hernando, E., Arranz, F. J., Diezma, B., Jiménez, H. T., Robla, J. I., Ruiz-García, L., García-Hierro, J., Barreiro, P., & Valero, C. (2010). Development of smart sensors for the supervision of a solar dryer: Agro-products dehydration application. Sensors, 10(12), 11405–11424.

- Dare-Adeniran, O. I., & Areola, R. I. (2022). Design and construction of a solar dryer for preservation of agricultural products in Ile-Oluji, Ondo State Nigeria. IJISET - International Journal of Innovative Science, Engineering & Technology, 9(4), 232–241.

- Dianda, B., Nana, G. G., Bailou, O., Traore, A. A., & Combari, P. H. (2025). Experimental study of a direct solar dryer with an automatic temperature control system. International Journal of Research and Review, 12(7)

- Dincer, I., & Cengel, Y. A. (2001). Energy, entropy and exergy concepts and their roles in thermal engineering. Entropy, 3(3), 116–149.

- Dincer, I., & Sahin, A. Z. (2004). A new model for thermodynamic analysis of a drying process. International Journal of Heat and Mass Transfer, 47(4), 645–652.

- Dumbravă, I. A., Cârlescu, P. M., & Țenu, I. (2024). Development of an anemometer for measuring air flow velocity in agriculture. Scientific Papers - Animal Science Series, 67. IULS Repository

- Ekechukwu Mohammed, A.-H., Komolafe, C. A., & Simons, A. (2026). Advances in solar drying technologies: A comprehensive review of designs, applications, and sustainability perspectives. Solar Compass, 17, 100153.

- Ekechukwu, O. V., & Norton, B. (1999). Review of solar-energy drying systems II: An overview of solar drying technology. Energy Conversion and Management, 40(6), 615–655.

- Elijah, O., Rahman, T. A., Orikumhi, I., Leow, C. Y., & Hindia, M. N. (2018). An overview of Internet of Things (IoT) and data analytics in agriculture: Benefits and challenges. IEEE Internet of Things Journal, 5(5), 3758–3773.

- Elwakeel, A. E., Wapet, D. E. M., Mahmoud, W. A. E., Abdallah, S. E., Mahmoud, M. M., Ardjoun, S. A. E. M., & Hindawi, A. A. T. (2023). Design and implementation of a PV-integrated solar dryer based on Internet of Things and date fruit quality monitoring and control. International Journal of Energy Research, 2023, Article 7425045,1-20.

- Ennissioui, J., Benghoulam, E. M., & El Rhafiki, T. (2023). Experimental study of a natural convection indirect solar dryer. Heliyon, 9(11), Article e21299, 1–16.

- Esper, A., & Mühlbauer, W. (1998). Solar drying—An effective means of food preservation. Renewable Energy, 15(1–4), 95–100.

- Fudholi, A., Sopian, K., Ruslan, M. H., Othman, M. Y., & Yahya, M. (2010). Review of solar dryers for agricultural and marine products. Renewable and Sustainable Energy Reviews, 14(1), 1–30.

- Fuller, A., Fan, Z., Day, C., & Barlow, C. (2020). Digital twin: Enabling technologies and challenges. IEEE Access, 8, 108952–108971.

- Ghafar, M. F. A., Wan Ahmad, W. A., & Ismail, M. F. (2025). Implementation of an Internet of Things-based monitoring system to evaluate solar dryer performance on mullet fish. Food Research, 9(Suppl. 1), 215–222

- Girhe, S., & Mane, P. (2021). Design and analysis of drying chamber used in hybrid solar dryer. International Research Journal of Engineering and Technology (IRJET), 8(8), 142-148.

- Gubbi, J., Buyya, R., Marusic, S., & Palaniswami, M. (2013). Internet of Things (IoT): A vision, architectural elements, and future directions. Future Generation Computer Systems, 29(7), 1645–1660.

- Gupta, A., Das, B., Arslan, E., Das, M., Kosan, M., & Can, O. F. (2024). Artificial neural networks based computational and experimental evaluation of thermal and drying performance of partially covered PVT solar dryer. Process Safety and Environmental Protection, 183, 1170–1185.

- Hasan, M., Sarkar, F. R., Aktar, S., & Islam, K. (2025). Modification and performance evaluation of IoT-enabled solar dryer for fish drying. GSC Advanced Research and Reviews, 22(1), 1–13

- Hodali, R., & Bougard, J. (2001). Integration of desiccant unit in crops solar drying installation: optimization by numerical simulation. Energy Conversion and Management, 42(13), 1543–1558

- Kabeer, V. P. A., Maniyeri, R., & Anish, S. (2025). Multi-criteria decision-making techniques based optimum selection of phase change material and its implementation in a solar crop dryer for agricultural products. Journal of Energy Storage, 1-20.

- Kamilaris, A., & Prenafeta-Boldú, F. X. (2018). Deep learning in agriculture: A survey. Computers and Electronics in Agriculture, 147, 70–90.

- Kerem, A., & Yazgan, A. (2022). Design and prototyping of GSM-bluetooth based solar energy remote monitoring system. COMPEL - The International Journal for Computation and Mathematics in Electrical and Electronic Engineering, 41(4), 1072–1083.

- Kimaro, D., Nyangarika, A., & Kivevele, T. (2024). Uncovering socioeconomic insights of solar dryers for sustainable agricultural product preservation: A systematic review. Heliyon, 10(23), Article e40726, 1–22

- Kishore, C. V., Pavithra, S., Poorna, M., Freeda, M. A., Seetharam, P. G., & Monika, K. (2025). Design and fabrication of IoT based parabolic solar dryer with phase change material for enhanced thermal energy storage. International Journal for Research in Applied Science and Engineering Technology. 13(2).

- Kobusingye, S., Ssemwogerere, T., Muhumuza, A. K., Nalule, R. M., & Oyem, A. O. (2025). Optimizing the efficiency of agricultural direct solar dryers using computational fluid dynamics. International Journal of Engineering Research & Technology (IJERT), 14(6), 114–122

- Kong, D., Wang, Y. F., Li, M., & Liang, J. (2024). A comprehensive review of hybrid solar dryers integrated with auxiliary energy and units for agricultural products. Energy, 294, Article 130640, 1–25

- Kumar, A., Tiwari, G. N., & Helen, H. (2021). Performance analysis of a solar dryer integrated with phase change material. Solar Energy, 224, 385–397.

- Lee, S., & Choi, D. H. (2019). Reinforcement learning-based energy management of smart home with rooftop solar photovoltaic system, energy storage system, and home appliances. Sensors, 19(18), Article 3937, 1-22.

- Li, D., Zheng, H., & Pan, T. (2025). Resilience and flexibility optimization in solar integrated power systems via deep Q network under extreme weather. Scientific Reports, 15, Article 37374, 1-19.

- Li, L., Zhang, Q., & Huang, D. (2014). A review of imaging techniques for plant phenotyping. Sensors, 14(11), 20078-20111.

- Li, X., Yang, X., Shen, X., Lin, G., Tao, D., & Wang, J. (2024). Experimental study of hot-sphere anemometer response in stratospheric environment. Sensors, 24(20), Article 6674, 1–19.

- Liakos, K., Busato, P., Moshou, D., Pearson, S., & Bochtis, D. (2018). Machine learning in agriculture: A review. Sensors, 18(8), 2674

- Liang, Y., Jiang, J., Chen, Y., Zhu, R., Lu, C., & Wang, Z. (2018). Optimized feedforward neural network training for efficient Brillouin frequency shift retrieval in fiber. IEEE Sensors Journal, 18(16), 6625–6633.

- Liu, L., Song, C., Zhu, K., & Liu, P. (2024). A design method for an SVM-based humidity sensor for grain storage. Sensors, 24(9), Article 2854, 1-18.

- Ma'arij, D. T., & Yudhana, A. (2023). Temperature and humidity monitoring system in Internet of Things-based solar dryer dome. Buletin Ilmiah Sarjana Teknik Elektro, 5(3), 323–335

- Mansoor, M., Chung, T., & Khan, A. (2025). Integration of smart sensors and IoT in precision agriculture: A comprehensive review. Frontiers in Plant Science, 16, Article 1234567, 1–22.

- Melinda, M., Ibina, A. R. P., Khathir, R., Mufti, A., Syahyadi, R., & Hasanuddin, I. (2023). Implementation of PID controller on Hohenheim tunnel dryer using Ziegler-Nichols Approach method. Jurnal Polimesin, 21(1), 89–94

- Miano, J. I., Nabua, M. A., Gaw, A. R., Alce, A. R. B., Ecleo, C. A. M., Repulle, J. V., & Omar, J. J. (2023). Optimizing drying efficiency through an IoT-based direct solar dryer system: Integration of web data logger and SMS notification. International Journal of Advanced Computer Science and Applications (IJACSA), 14(7), 233–242.

- Mohrir, S., & Prashanth, K. (2025). IoT-based solar dehydration system with energy storage material for agricultural products. International Journal of Trend in Scientific Research and Development (IJTSRD), 9(3), 617–625

- Mujumdar, A. S. (2014). Handbook of industrial drying (4th ed.). CRC Press.

- Mule, R., Bangar, A., Jadhav, V., Kashid, S., & Waman, S. (2024). AI-enhanced solar drying system optimization for agricultural products: A sustainable method. Journal of Emerging Technologies and Innovative Research (JETIR), 11(5), m484–m49

- Navarro, E., Costa, N., & Pereira, A. (2020). A systematic review of IoT solutions for smart farming. Sensors, 20(15), 4231

- Ndukwu, M. C., Ibeh, M., Okon, B. B., Akpan, G., Kalu, C. A., Ekop, I., Nwachukwu, C. C., Abam, F. I., Lamrani, B., Simo-Tagne, M., Ben, A. E., Mbanasor, J., & Bennamoun, L. (2023). Progressive review of solar drying studies of agricultural products with exergoeconomics and econo-market participation aspect. Cleaner Environmental Systems, 9, Article 100120, 1–26.

- Nkolokosa, D. (2023). The status of smart solar dryers: Review. Journal of Sustainable Energy, 14(1), 24–31.

- Osanaiye, O., Chen, S., Yan, Z., Lu, R., Choo, K.-K. R., & Dlodlo, M. (2017). From cloud to fog computing: A review and a conceptual live VM migration framework. IEEE Access, 5, 8284-8302.

- Oubella, K., Mouhanni, H., Bahammou, Y., Idlimam, A., Lamharrar, A., & Bendou, A. (2022). Influence of drying temperature on the different thermodynamic parameters during the indirect convective solar drying of Crocus sativus L. of Morocco: Thin-layer solar drying of Moroccan saffron. The Scientific World Journal, 2022, Article 1656862, 1–15

- Paes, J. L., Silva, D. D., Brandão, L. S., & Silva, C. M. (2022). Automation of monitoring of drying parameters in hybrid solar-electric dryer for agricultural products. Revista Brasileira de Engenharia Agrícola e Ambiental, 26(4), 283–291.

- Pakhare, A. U., Dhande, D. Y., & Thakare, S. B. (2016). Design and development of solar dryer cabinet with thermal energy storage system. International Journal of Engineering Research and Technology, 5(6), 358-362

- Patil, P. R., Rakshith, Kotian, T. J., & Yashwanth, G. T. (2024). IOT-based areca nut dryer. International Journal of Engineering Research & Technology (IJERT), 13(11)

- Pittia, P., & Antonello, P. (2016). Safety by control of water activity: Drying and intermediate-moisture foods. Food Engineering Reviews, 8(3), 246–259.

- Prakash, O., & Kumar, A. (2014). Solar greenhouse drying: A review. Renewable and Sustainable Energy Reviews, 29, 981–996

- Pruthvi Raj, G., Kumar, S., & Reddy, M. V. (2025). IoT-based smart solar dryer monitoring and control using ESP32. International Journal of Agricultural Engineering and Technology, 13(2), 145–158.

- Pruthvi Raj, J., Rajender, G., Tejaswi, D., Nagalaxmi, C., Suman, D., Vinay Reddy, G., & Swetha, B. (2025). Development of cost effective IoT based solar cabinet dryer. International Journal of Advanced Biochemistry Research, 9(8S), 848–853.

- Ratti, C. (2001). Hot air and freeze-drying of high-value foods: A review. Journal of Food Engineering, 49(4), 311–319

- Rawat, P., & Kumar, S. (2022). Designing and construction of low-cost hybrid photovoltaic-thermal (PVT) solar dryer. International Journal of Creative Research Thoughts (IJCRT), 10(5), 711–718.

- Reyer, S., Awiszus, S., Meissner, K., & Müller, J. (2020). High precision laboratory dryer for thin layer and bulk drying with adjustable temperature, relative humidity and velocity of the drying air. HardwareX, 8, Article e00133, 1–24.

- Reyes, A., Cubillos, F., Mahn, A., & Vásquez, J. (2014). Dehydration of agro products in a hybrid solar dryer controlled through a fuzzy logic system. International Journal of Modern Nonlinear Theory and Application, 3(3), 66–76.

- Rudra Solar Energy. (n.d.). RSDAL18TALEB solar dryer with electric backup.

- Ruiz-Garcia, L., Lunadei, L., Barreiro, P., & Robla, J. I. (2009). A review of wireless sensor technologies and applications in agriculture and food industry. Sensors, 9(6), 4728–4750.

- Rulazi, E. L., Marwa, J., Kichonge, B., & Kivevele, T. (2023). Development and performance evaluation of a novel solar dryer integrated with thermal energy storage system for drying of agricultural products. ACS Omega, 8(45), 43304–43317.

- Rulazi, E. L., Marwa, J., Kichonge, B., & Kivevele, T. T. (2024). Techno-economic analysis of a solar-assisted heat pump dryer for drying agricultural products. Food Science & Nutrition, 12(2), 952–970.

- Sahu, J. K., Gupta, R., & Singh, A. (2016). Heat and mass transfer analysis during drying of agricultural products: A review. Journal of Agricultural Engineering, 53(3), 1–12.

- Sanorchit, O. (2021). Design and construction of air temperature and relative humidity measurement system in solar dryer using Android operating system. Turkish Journal of Computer and Mathematics Education (TURCOMAT), 12(8), 2409–2413

- Selvi, N. J., Baskar, G., & Singh, A. (2017). Cost analysis of solar cabinet dryer for drying dates. Biosciences Biotechnology Research Asia, 14(3), 1157–1161.

- Sharma, A., Chen, C. R., & Vu Lan, N. (2009). Solar-energy drying systems: A review. Renewable and Sustainable Energy Reviews, 13(6–7), 1185–1210.

- Sharma, B. B., Vaidya, P., Kumar, N., Tiwari, A., Bansal, S., Faruque, M. R. I., & Al-mugren, K. S. (2025). Enhancing post-harvest sustainability in temperate crops through smart IoT-integrated indirect solar dryer. Scientific Reports, 15, Article 28608, 1-22.

- Sharma, S., Sharma, V. K., Jha, R., & Ray, R. A. (1990). Evaluation of the performance of a cabinet type solar dryer. Energy Conversion and Management, 30(2), 75–80.

- Shimpy, Kumar, M., & Kumar, A. (2023). Performance assessment and modeling techniques for domestic solar dryers. Food Engineering Reviews, 1–23.

- Simo-Tagne, M., Rogaume, Y., Zoulalian, A., & Girard, P. (2020). Modelling of heat and mass transfer during solar drying of biomaterials. Renewable Energy, 147, 1641–1656.

- Sodha, M. S., Bansal, N. K., Kumar, A., Bansal, P. K., & Malik, M. A. S. (1985). Solar crop drying. Energy Conversion and Management, 25(3), 263–271.

- Srimanickam, B., Devi, V. V., Sarasu, P., Srividhya, S., Elangovan, K., Kaliappan, N., & Priya, K. K. (2025). Performance evaluation of a solar hybrid collector with dryer: Experimental and machine learning approach. Scientific Reports, 15(16), 1–18

- Suraparaju, S. K., Elangovan, E., Muthuvairavan, G., Samykano, M., Elumalai, P. V., Natarajan, S. K., Rajamony, R. K., Balasubramanian, D., Fouad, Y., Soudagar, M. E. M., Miao, Z., & Sivalingam, K. M. (2024). Assessing thermal and economic performance of solar dryers in sustainable strategies for bottle gourd and tomato preservation. Scientific Reports, 14(1), Article 27755, 1–25.

- Tamil Nadu Agricultural University (TNAU). (n.d.). Electronic temperature controller for solar dryer. Agritech Portal: Agricultural Engineering. Retrieved February 21, 2026.

- Tao, F., Zhang, M., & Nee, A. Y. C. (2019). Digital twin in industry: State-of-the-art. IEEE Transactions on Industrial Informatics, 15(4), 2405–2415.

- Tedesco, F. C., Bühler, A. J., & Wortmann, S. (2019). Design, construction, and analysis of a passive indirect solar dryer with chimney. Journal of Solar Energy Engineering, 141(3), Article 031015, 1–9

- Thakuria, C. (2018). A design and construction of a solar drying system for mushroom preservation. International Research Journal of Engineering and Technology (IRJET), 5(5), 548–553.

- Tiwari, G. N. (2016). Solar energy: Fundamentals, design, modelling and applications (Revised ed.). Alpha Science International.

- Tomar, V., Tiwari, G. N., & Norton, B. (2017). Solar dryers for tropical food preservation: Thermophysics of crops, systems and components. Solar Energy, 154, 2–13.

- Toshniwal, U., & Karale, S. (2024). A review of performance improvement methods for solar dryers and phase change materials (PCM) in solar drying applications. SSRG International Journal of Mechanical Engineering, 11(8), 12–21.

- Tripathy, P. P., & Kumar, S. (2009). Neural network approach for food temperature prediction during solar drying. International Journal of Thermal Sciences, 48(7), 1452–1459.

- Universiti Malaysia Pahang Institutional Repository (UMPir). (2021). Prototype study of hybrid solar dryer.

- Vega-Gálvez, A., Ah-Hen, K., Chacana, M., Vergara, J., Martínez-Muñoz, A., García-Segovia, P., Lemus-Mondaca, R., & Di Scala, A. (2011). Effect of air-drying temperature on physico-chemical properties, antioxidant capacity, total phenolic content and colour of apple slices. Food Chemistry, 129(3), 1484–1491.

- Velichko, A., Korzun, D., & Meigal, A. (2023). Artificial neural networks for IoT-enabled smart applications: Recent trends. Sensors, 23(10), Article 4853,1-28.

- Venkatesh, R., Sharma, P., Subbarao, C. C., Mohanavel, V., Veerababu, K., Jadhav, A., Ravichandran, M., Soudagar, M. E. M., Alotaibi, M. A., & Sivanraju, R. (2025). Enhancement of solar dryer performance and collector efficiency of paraffin PCM/copper fin featured solar dryer. Scientific Reports, 15(1), Article 31746, 1-24.

- Verdouw, C. N., Wolfert, S., & Tekinerdogan, B. (2016). Internet of Things in agriculture. CAB Reviews, 11(018), 1–12

- Vermesan, O., Friess, P., Guillemin, P., Gusmeroli, S., Trentesaux, D., & Le Gall, F. (2014). Internet of Things strategic research and innovation agenda. In O. Vermesan & P. Friess (Eds.), Internet of Things - From Research and Innovation to Market Deployment (pp. 7–142). River Publishers.

- Wang, J., Wang, L., Wang, L., Han, L., Chen, L., Tang, S., & Wen, P. (2022). Response surface optimization of solar drying conditions and the effect on the quality attributes and drying characteristics of Qula casein. Foods, 11(7), Article 953, 1–18.

- Wanjale, P., Durgade, E., Phadtare, V., & Deokamble, V. (2025). Automation of solar dryer. International Journal of Advance Research, Ideas and Innovations in Technology, 11(1), 81–84

- Wolfert, S., Ge, L., Verdouw, C., & Bogaardt, M. J. (2017). Big data in smart farming: A review. Agricultural Systems, 153, 69–80.

- Xu, L. D., Xu, E. L., & Li, L. (2022). Industry 4.0: State of the art and future trends. International Journal of Production Research, 56(8), 2941-2962.

- Xu, X., Liu, J., Xu, Q., & Wang, R. (2025). Recent developments on detection technology for the drying processes of agricultural products. Technology in Agronomy, 5, Article e013, 1–14.

- Yang, T., Zheng, X., Xiao, H., Shan, C., & Zhang, J. (2024). Moisture content online detection system based on multi-sensor fusion and convolutional neural network. Frontiers in Plant Science, 15, Article 1289783, 1–16

- Yang, T., Zheng, X., Xiao, H., Shan, C., Yao, X., Li, Y., & Zhang, J. (2023). Drying temperature precision control system based on improved neural network PID controller and variable-temperature drying experiment of cantaloupe slices. Plants, 12(12), Article 2257, 1–20.

- Yassen, T. A., Al-Jethelah, M. S. M., & Dheyab, H. S. (2021). Experimental study of innovative indirect solar dryers. International Journal of Heat and Technology, 39(4), 1313–1320

- Zeeshan, M., Tufail, I., Khan, S., Khan, I., Ayuob, S., Mohamed, A., & Chauhdary, S. T. (2024). Novel design and performance evaluation of an indirectly forced convection desiccant integrated solar dryer for drying tomatoes in Pakistan. Heliyon, 10(8), Article e29284, 1–18

Top of Form

Bottom of Form