Original Research Title page

ANN-assisted prediction of groundwater WQI for drinking-water screening in Salur Mandal, Andhra Pradesh, India

## ABSTRACT

Aim:
Groundwater quality monitoring in hard-rock aquifer systems is essential for sustainable resource management. This study aims to assess groundwater quality using the weighted arithmetic Water Quality Index (WQI) method and to develop an Artificial Neural Network (ANN) model for reliable prediction of WQI in the Salur region of Andhra Pradesh, India.

Study Design: A quantitative analytical study integrating WQI assessment with predictive modeling was conducted using field-monitored groundwater quality data collected over a three-year period (November 2018 to October 2021).

Methodology:
A total of 468 groundwater samples was collected from 13 monitoring stations between November 2018 and October 2021 and analyzed for fifteen physicochemical parameters adopting standard procedures prescribed by APHA 2017. WQI was computed using the weighted arithmetic method to classify groundwater suitability. A feed-forward ANN model with a 15–5–1 architecture was developed using the Levenberg–Marquardt algorithm. The dataset was randomly divided into 70% training, 15% validation, and 15% testing subsets using MATLAB toolbox.

Results:
WQI results revealed that 77% of groundwater samples fall within the good category (WQI = 26–50), while 23% fall under the Poor category (WQI = 51–75), indicating moderate hydro chemical variability without critical contamination. The ANN model demonstrated high predictive accuracy, with Root Mean Square Error (RMSE=0.438) and coefficient of determination (R²) of 0.998

Conclusion:
The developed ANN model effectively predicts groundwater WQI with high accuracy and can serve as a robust decision-support tool for sustainable groundwater management in hard-rock aquifer regions, particularly in data-scarce rural areas.

Keywords: Water Quality Index; Artificial Neural Network; hard-rock aquifer; groundwater sustainability; Levenberg–Marquardt; India

# 1. INTRODUCTION

Groundwater constitutes the primary source of drinking water in rural India, particularly in hard-rock aquifer regions where surface water availability is limited. Its quality is governed by lithology, recharge dynamics, mineral dissolution, and anthropogenic influences (Gao et al., 2020; Kumar el al., 2021; Subba Rao et al., 2020; Wu et al., 2019).

Water Quality Index (WQI) methods provide simplified representation of hydrochemical status by integrating multiple parameters into a single indicator. The weighted arithmetic WQI approach, originally conceptualized by Horton (1965) and later refined by Brown et al. (1972), has been widely applied for groundwater suitability assessment.

In India, Previous studies have reported groundwater quality assessment using WQI in various Indian regions (Ramakrishnaiah, et al., 2009; P. N. Rajankar et al., 2009; Vasanthavigar et al., 2010; Swarna Latha et al 2010; Srinivasa Rao & Nageswararao, 2013; Gummadi et al 2014; Pavan kumar et al., 2017a; Nayar,2020; Muthukumaran et al.,2022). However, conventional WQI approaches remain descriptive and lack predictive capability.

Recent advancements in deep learning have enabled robust groundwater quality forecasting (Basin et al 2018; Liv et al 2022; Li et al., 2023; Singh, et al., 2024; Chen et al 2025). Machine learning applications for water quality prediction have expanded recently (Maier & Dandy, 2000; Pavan Kumar et al., 2017b; Muthukumaran & Vinoth Kumar, 2022; Ghadai et al., 2022; Rabah Ismail et al., 2024; García-del-Toro et al., 2025). Statistical modelling of groundwater-quality indicators based on hydrochemical parameter relationships has been documented in recent literature (Basavaraja et al., 2025). Recent work has increasingly coupled WQI/GWQI/GQI frameworks with modern AI/ML (including neural networks, Bayesian neural networks, and hybrid deep-learning architectures) to improve index prediction, identify key controlling variables, and support spatial decision-making for drinking-water screening (Sajib et al., 2023; Ahn et al., 2023; Maiti et al., 2024; Ehteram et al., 2024). For example, ANN-based and related neural-network approaches have been used to predict groundwater quality indices and integrate broader chemical information for potable-water suitability screening (Sajib et al., 2023; Ahn et al., 2023), while Bayesian neural networks have been applied to both predict GWQI and identify the most influential parameters (Maiti et al., 2024). Hybrid/advanced deep-learning models have also shown strong performance for WQI prediction in recent studies (Ehteram et al., 2024), and comprehensive reviews synthesize best practices, limitations (e.g., data leakage and validation design), and emerging directions such as explainable AI for groundwater quality modeling (Haggerty et al., 2023). ANN models are especially advantageous in data-scarce hydro geological settings due to their capacity to approximate complex functional mappings without explicit physical equations.

Despite increasing ANN applications, predictive WQI modeling remains limited in inland hard-rock aquifer systems of rural India. Therefore, this study aims to:

- Assess groundwater quality using weighted arithmetic WQI

- Develop and validate an ANN-based predictive model for monthly WQI estimation

- Evaluate model robustness and performance

This integrated approach contributes toward proactive and data-driven groundwater sustainability management.

# 2. MATERIALS AND METHODS

## 2.1 Study Area

The study area is located (Fig.1) in Salur Mandal (currently Parvathipuram Manyam District), Andhra Pradesh, India, covering ~328 km² between 18°28′12″–18°46′48″N and 83°01′12″–83°17′24″E. The region is characterized by hard-rock aquifers dominated by Khondalite Group formations of the Eastern Ghats. The climate is tropical monsoonal, with seasonal recharge influencing groundwater chemistry. Groundwater serves as the principal drinking water source for rural and tribal communities (K. Krishna Kumar 2012).

Fig. 1: Location map of the study area (Appala Raju et al., 2014)

## 2.2 Sampling and Laboratory Analysis

A total of 468 groundwater samples (monthly each sample from each sampling location for 36 months) were collected from November 2018 to October 2021 from thirteen monitoring locations in the study area. Geological coordinates of the sampling locations are presented in Table 1. Samples were collected in pre-cleaned polyethylene bottles washed with non-ionic detergent, rinsed with tap water, treated with 1:1 hydrochloric acid, and finally rinsed with deionized water. Prior to sample collection, the bottles were rinsed three times with the respective groundwater. Tube wells were pumped for at least five minutes before sampling to obtain representative aquifer water.

Physico-chemical parameters were analyzed following standard procedures prescribed by APHA (1998). PH and electrical conductivity (EC) were measured using digital pH meter (Elico LI-120) and conductivity meter (Elico CL-351), respectively. Total dissolved solids (TDS) were determined by the gravimetric method. Total hardness (TH), total alkalinity (TA), calcium (Ca²⁺), magnesium (Mg²⁺), chloride (Cl⁻), carbonate (CO₃²⁻), and bicarbonate (HCO₃⁻) were estimated by standard titrimetric methods. Nitrate (NO₃⁻) was determined using a UV–Visible spectrophotometer (Elico SL-177) with a 1 cm quartz cell by the phenol disulphonic acid method. Fluoride (F⁻) was analyzed by the SPADNS method, while sulphate (SO₄²⁻) was determined by the turbidimetric method using standard barium chloride solution. Sodium (Na⁺) and potassium (K⁺) were measured using a flame photometer (Elico CL-361).

A total of fifteen physico-chemical parameters (pH, EC, TDS, TH, TA, Ca²⁺, Mg²⁺, Na⁺, K⁺, NO₃⁻, Cl⁻, F⁻, SO₄²⁻, CO₃²⁻, and HCO₃⁻) were utilized to compute the Water Quality Index (WQI) using the weighted arithmetic method.

Table 1: Groundwater sampling locations and coordinates

| Sample Id | Sampling Station | Mandal | Latitude(N) | Longitude(E) |
| --- | --- | --- | --- | --- |
| S1 | Annamarajuvalasa | Salur | 18°37′46″ | 83°11′04″ |
| S2 | Borabanda | Salur | 18°33′35″ | 83°14′31″ |
| S3 | Chinavootagedda | Salur | 18°42′05″ | 83°11′43″ |
| S4 | Jilleduvalasa | Salur | 18°43′16″ | 83°08′07″ |
| S5 | Kandulapalem | Salur | 18°38′40″ | 83°09′22″ |
| S6 | Karasuvalasa | Salur | 18°34′34″ | 83°10′05″ |
| S7 | Kurukutti | Salur | 18°35′20″ | 83°05′35″ |
| S8 | Maripalle | Salur | 18°37′54″ | 83°09′07″ |
| S9 | Mirtivalasa | Salur | 18°36′4″ | 83°13′44″ |
| S10 | Neliparti | Salur | 18°30′16″ | 83°10′57″ |
| S11 | Parannavalasa | Salur | 18°32′22″ | 83°15′11″ |
| S12 | Puroithunivalasa | Salur | 18°34′08″ | 83°14′33″ |
| S13 | Sivarampuram | Salur | 18°32′11″ | 83°14’36” |

2.3 Water Quality Index

Several Water Quality Index (WQI) models have been proposed by researchers, notably Horton (1965) and Brown et al. (1972), and have since been widely adopted by governmental agencies and researchers for water quality assessment. Among these, the Weighted Arithmetic Water Quality Index (WAIWQI) method is one of the most popular and extensively applied approaches. The WQI was computed using the below equations (Eq.1- Eq. 4).

WQI=QiWiWi        (Eq. 1)

The quality rating scale (Qi) for each parameter is calculated by using this expression:

Qi=100Vi-VoSi-Vo     (Eq. 2)

Where, Vi= estimated concentration of ith parameter in the analysed water, Vo=The ideal value of this parameter in pure water

Vo=0 (except pH =7.0)

Si=Standard value of ithparameter

The unit weight (Wi) for each water quality parameter is calculated by using the following formula:

Wi=KSi             (Eq. 3)

Where, K=proportionality constant and can also be calculated by using the following equation:

# K=11Si         (Eq. 4)

Using the equations, WQI values were computed for all sampling locations and periods. Based on the obtained WQI values, the groundwater quality was classified into standard rating categories to interpret its suitability for drinking purposes. It is presented in Table 2.

Table 2: Water Quality rating as per WA-WQI model

Brown et al (1972)]

| WQI Value | Rating of Water Quality | Suitability for drinking |
| --- | --- | --- |
| 0-25 | Excellent | Fit for drinking |
| 26-50 | Good | Acceptable for drinking |
| 51-75 | Poor | Moderately polluted |
| 76-100 | Very Poor | Excessively polluted |
| Above 100 | Unsuitable for drinking purpose | Unfit for drinking |

## 2.4 Artificial Neural Network (ANN) Model development for prediction of WQI

A feed-forward Multilayer Perceptron (MLP) Artificial Neural Network (ANN) was developed to predict monthly Water Quality Index (WQI) values using hydro chemical parameters as inputs. The dataset comprised out of 468 samples, 390 monthly groundwater samples from 13 monitoring stations over a 30-month period taken as training and validation period. A period six of months was considered as testing period. The dataset was randomly divided using MATLAB’s random split default dividerand function into training (70%, n = 273), validation (15%, n = 59), and testing (15%, n = 58) subsets. The validation subset was utilized for early stopping to control overfitting during training, whereas the testing subset was reserved exclusively for independent model evaluation.

Fifteen physicochemical parameters—pH, EC, TDS, Ca²⁺, Mg²⁺, Na⁺, K⁺, Cl⁻, SO₄²⁻, HCO₃⁻, CO₃²⁻, NO₃⁻, F⁻, Total Hardness (TH), and Total Alkalinity (TA) —served as input neurons, and the calculated WQI was treated as the single output neuron. Prior to model development, all input variables were normalized to the range of 0–1 using min–max scaling to enhance training efficiency and numerical stability.

The optimized ANN architecture (Fig.2) consisted of 15 input neurons, one hidden layer containing 5 neurons, and one output neuron (15–5–1 configuration).  A tangent sigmoid (tansig) activation function was employed in the hidden layer to model nonlinear relationships among the hydro chemical variables, while a linear (purelin) function was applied at the output layer to generate continuous WQI predictions.

The selection of hidden neurons was performed through iterative trials by testing networks with 3 to 10 neurons and comparing validation errors. The configuration chose with five hidden neurons the best balance between prediction accuracy and generalization performance. Increasing the number of neurons beyond five did not significantly reduce validation error and posed a greater risk of overfitting.

Training was carried out using the Levenberg–Marquardt back propagation algorithm (trainlm) available in the Neural Network Toolbox of MATLAB (R2023a, Math Works Inc., USA), chosen for its rapid convergence with moderate-sized datasets. The final model comprised 86 adjustable parameters, which is substantially lower than the number of training samples (n = 273), thereby ensuring statistical adequacy and limiting over-parameterization.

Model performance was assessed using the coefficient of determination (R²), root mean square error (RMSE), mean absolute error (MAE), and mean bias error (MBE), to comprehensively evaluate model accuracy and predictive capability. The performance indicators are calculated using below equations.

### Model Performance Evaluation indicators

Model performance was evaluated using the equations (Eq.5 – Eq. 8) presented below.

### Coefficient of Determination

R² = 1 − [Σ(Oi − Pi)² / Σ(Oi − Ō)²]     (Eq. 5)

### Root Mean Square Error

RMSE = √[(1/n) Σ(Oi − Pi)²]          (Eq. 6)

### Mean Absolute Error

MAE = (1/n) Σ|Oi − Pi|           (Eq. 7)

Mean Bias Error

MBE= (1/n) Σ(Pi – Oi)    (Eq. 8)

where
Oi = observed WQI; Pi = predicted WQI; Ō = mean observed WQI; n = number of samples

Figure 2: Architecture of the feed-forward ANN model used for WQI prediction

# RESULTS AND DISCUSSION

## 3.1.  Physico chemical Analysis

## Fifteen physicochemical parameters of all samples were measured using standard methods prescribed by APHA 2017. Parameters Min-Max values and Permissible values (Si) were presented in Table 3.

## Table 3: Physico-Chemical Parameters & Its Permissible values

| Sample ID | S1 |  | S2 |  | S3 |  | S4 |  | S5 |  | S6 |  | S7 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameters | Min | Max | Min | Max | Min | Max | Min | Max | Min | Max | Min | Max | Min | Max |
| pH | 7.05 | 7.93 | 7.05 | 7.53 | 7.35 | 7.93 | 7.05 | 7.53 | 7.05 | 7.93 | 7.45 | 8.03 | 7.35 | 8.03 |
| EC | 1249 | 1311 | 1110 | 1193 | 1499 | 1694 | 1168 | 1232 | 1335 | 1460 | 1792 | 1858 | 1327 | 1374 |
| TDS | 819 | 859 | 730 | 784 | 979 | 1104 | 768 | 808 | 916 | 954 | 1167 | 1209 | 869 | 899 |
| NO3 | 15 | 20.6 | 19 | 24.6 | 30.1 | 35.6 | 26 | 31.6 | 28 | 33.6 | 26 | 31.6 | 4 | 9.6 |
| TH | 365 | 388 | 355 | 383 | 359 | 378 | 335 | 368 | 285 | 318 | 460 | 488 | 350 | 378 |
| Ca | 33 | 39.6 | 38 | 46.6 | 31 | 37.6 | 31 | 35.6 | 36 | 41.6 | 51 | 56.6 | 35 | 40.6 |
| Mg | 69 | 73.6 | 82 | 88.6 | 64 | 71.6 | 63 | 68.6 | 44 | 51.6 | 79 | 83.6 | 62 | 65.6 |
| Na | 19 | 28.2 | 20 | 27.2 | 42 | 48.2 | 18 | 26.2 | 53 | 59.2 | 32 | 39.2 | 25 | 32.2 |
| K | 3.7 | 4.66 | 3.9 | 4.66 | 8.1 | 8.66 | 3.3 | 4.26 | 10.3 | 10.9 | 6.1 | 7.06 | 4.9 | 5.66 |
| Cl | 32 | 40.8 | 32 | 40.8 | 61 | 73.8 | 30 | 37.8 | 80 | 89.8 | 50 | 59.8 | 40 | 48.8 |
| SO4 | 20 | 26.6 | 31 | 39.6 | 54 | 60.6 | 10 | 15.6 | 40 | 45.6 | 42 | 47.6 | 33 | 38.6 |
| CO3 | 54 | 62.6 | 93 | 98.6 | 9 | 16.6 | 45 | 54.6 | 9 | 16.6 | 61 | 70.6 | 21 | 29.6 |
| HCO3 | 270 | 316 | 270 | 316 | 300 | 346 | 270 | 316 | 250 | 296 | 320 | 376 | 290 | 326 |
| F | 0.29 | 0.55 | 0.33 | 0.57 | 0.34 | 0.49 | 0.31 | 0.45 | 0.39 | 0.64 | 0.33 | 0.46 | 0.39 | 0.55 |
| TA | 270 | 336 | 280 | 326 | 310 | 376 | 280 | 326 | 270 | 316 | 340 | 396 | 310 | 346 |

| Sample ID | S8 |  | S9 |  | S10 |  | S11 |  | S12 |  | S13 |  | Permissible values |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameters | Min | Max | Min | Max | Min | Max | Min | Max | Min | Max | Min | Max |  |
| pH | 7.15 | 7.65 | 7.55 | 8.13 | 7.15 | 7.73 | 6.95 | 7.53 | 7.35 | 7.93 | 7.45 | 8.03 | 8.5 |
| EC | 1022 | 1076 | 1428 | 1481 | 1970 | 2038 | 1392 | 1460 | 2240 | 2311 | 1620 | 2004 | 1000 |
| TDS | 674 | 709 | 934 | 969 | 1279 | 1324 | 911 | 954 | 1454 | 1499 | 1055 | 1302 | 600 |
| NO3 | 15 | 20.6 | 14 | 20.6 | 25 | 31.6 | 16 | 21.6 | 17 | 22.6 | 16 | 22.6 | 50 |
| TA | 295 | 318 | 365 | 403 | 395 | 423 | 415 | 438 | 595 | 643 | 415 | 438 | 500 |
| Ca | 26 | 31.6 | 59 | 65.6 | 57 | 63.6 | 38 | 44.6 | 64 | 68.6 | 40 | 48.6 | 200 |
| Mg | 52 | 58.6 | 51 | 56.6 | 59 | 64.6 | 74 | 80.6 | 111 | 117 | 73 | 81.6 | 150 |
| Na | 18 | 24.2 | 37 | 44.2 | 39 | 46.2 | 28 | 34.2 | 48 | 54.2 | 24 | 30.2 | 200 |
| K | 3.7 | 4.26 | 7.6 | 8.36 | 7.5 | 8.46 | 5.7 | 6.16 | 9.5 | 10.1 | 4.8 | 5.26 | 20 |
| Cl | 27 | 34.8 | 57 | 66.8 | 57 | 68.8 | 43 | 50.8 | 73 | 81.8 | 38 | 45.8 | 250 |
| SO4 | 25 | 34.6 | 34 | 39.6 | 14 | 19.6 | 82 | 87.6 | 9 | 15.6 | 23 | 29.6 | 250 |
| CO3 | 41 | 47.6 | 43 | 49.6 | 65 | 72.6 | 34 | 40.6 | 119 | 129 | 63 | 70.6 | 30 |
| HCO3 | 190 | 246 | 270 | 316 | 270 | 326 | 290 | 336 | 420 | 466 | 310 | 356 | 500 |
| F | 0.24 | 0.38 | 0.6 | 0.75 | 0.17 | 0.33 | 0.69 | 0.92 | 0.51 | 0.74 | 0.3 | 0.47 | 1.2 |
| TA | 210 | 266 | 280 | 336 | 280 | 346 | 310 | 366 | 430 | 486 | 320 | 376 | 200 |

Monthly WQI values were computed for all 468 groundwater samples collected between November 2018 and October 2021. The year wise average WQI values and their ratings prescribed by Brown et al 1972 are presented in Table 3.

## Water Quality Index Distribution

## Table 4:  WQI Values and their ranking

| Sample ID | Yearly Average WQI Values |  |  | Average WQI values | WQI ranking |
| --- | --- | --- | --- | --- | --- |
|  | Year 1 | Year 2 | Year 3 |  |  |
| S1 | 39.01 | 40.94 | 40.70 | 40.21 | Good |
| S2 | 42.96 | 44.89 | 44.65 | 44.17 | Good |
| S3 | 35.42 | 37.35 | 37.11 | 36.63 | Good |
| S4 | 33.73 | 35.66 | 35.42 | 34.94 | Good |
| S5 | 42.04 | 43.97 | 43.73 | 43.25 | Good |
| S6 | 40.42 | 42.35 | 42.11 | 41.63 | Good |
| S7 | 38.97 | 40.90 | 40.66 | 40.18 | Good |
| S8 | 28.84 | 30.77 | 30.53 | 30.04 | Good |
| S9 | 56.74 | 58.67 | 58.42 | 57.94 | Poor |
| S10 | 29.33 | 31.26 | 31.02 | 30.54 | Good |
| S11 | 60.51 | 62.44 | 62.20 | 61.72 | Poor |
| S12 | 61.48 | 63.41 | 63.17 | 62.69 | Poor |
| S13 | 38.75 | 40.68 | 40.44 | 39.95 | Good |

WQI result from Table 4 states that the majority of samples (77%) were classified as good, indicating overall acceptable groundwater quality for drinking purposes with minor treatment. A smaller proportion of samples (23%) fell within the Poor category, indicating localized deterioration likely associated with geogenic mineral dissolution, and source of contamination (particularly pre-monsoon). No samples were categorized under Very Poor or Unsuitable classes, indicating the absence of severe contamination during the monitoring period.

The observed WQI variability reflects nonlinear interactions among major cations, anions, and physicochemical parameters. Seasonal recharge and evapoconcentration likely influence fluctuations in TDS, hardness, and major ion chemistry, which in turn affect WQI values. However, the absence of extreme WQI classes (WQI 76-100, WQI >100) suggests that groundwater quality degradation is moderate rather than critical within the investigated region.

## 3.2. ANN Performance Evaluation

The comparison between experimentally computed and ANN-predicted Water Quality Index (WQI) values for six months (from May-2021 to October-2021) is presented in Table 5. The model demonstrated excellent predictive performance, with a high coefficient of determination (R² = 0.998), indicating a very strong agreement between observed and predicted values.

Table 5: Comparison Between Experimented and ANN Predicted WQI Values during Testing

| Sample ID | Month& Year → | May-2021 | Jun-2021 | Jul-2021 | Aug-2021 | Sep-2021 | Oct-2021 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S1 | Experimented | 48.41 | 48.77 | 37.03 | 35.52 | 34.52 | 31.62 |
|  | ANN Predicted | 48.41 | 48.98 | 36.28 | 35.23 | 34.36 | 30.94 |
| S2 | Experimented | 50.72 | 50.43 | 40.54 | 38.24 | 38.54 | 38.44 |
|  | ANN Predicted | 50.59 | 50.62 | 40.43 | 37.94 | 38.97 | 38.30 |
| S3 | Experimented | 40.75 | 40.78 | 34.23 | 34.13 | 32.37 | 35.49 |
|  | ANN Predicted | 40.97 | 41.00 | 34.26 | 34.12 | 32.15 | 35.54 |
| S4 | Experimented | 38.60 | 39.44 | 33.12 | 32.62 | 33.06 | 31.92 |
|  | ANN Predicted | 38.58 | 39.10 | 33.05 | 32.65 | 32.94 | 32.24 |
| S5 | Experimented | 51.66 | 49.14 | 40.24 | 36.38 | 35.13 | 38.19 |
|  | ANN Predicted | 50.43 | 49.21 | 40.02 | 36.32 | 35.10 | 37.85 |
| S6 | Experimented | 44.24 | 45.80 | 40.62 | 38.79 | 40.61 | 39.07 |
|  | ANN Predicted | 44.00 | 45.98 | 40.12 | 39.05 | 40.40 | 38.21 |
| S7 | Experimented | 43.80 | 45.04 | 39.71 | 38.10 | 36.21 | 36.36 |
|  | ANN Predicted | 43.72 | 45.19 | 39.66 | 38.14 | 36.76 | 35.98 |
| S8 | Experimented | 33.55 | 34.16 | 26.73 | 28.36 | 26.38 | 28.96 |
|  | ANN Predicted | 33.52 | 34.31 | 26.01 | 28.22 | 25.12 | 29.16 |
| S9 | Experimented | 62.08 | 62.67 | 55.98 | 56.36 | 54.72 | 53.64 |
|  | ANN Predicted | 62.18 | 62.82 | 56.13 | 56.09 | 54.42 | 53.53 |
| S10 | Experimented | 35.45 | 34.99 | 29.62 | 27.84 | 27.99 | 26.28 |
|  | ANN Predicted | 35.48 | 35.24 | 29.76 | 27.83 | 28.68 | 26.01 |
| S11 | Experimented | 67.74 | 68.14 | 56.22 | 55.15 | 56.75 | 54.51 |
|  | ANN Predicted | 67.32 | 67.85 | 57.80 | 55.13 | 56.04 | 54.42 |
| S12 | Experimented | 67.70 | 69.30 | 61.10 | 57.82 | 56.24 | 56.99 |
|  | ANN Predicted | 68.09 | 69.50 | 60.92 | 57.55 | 54.58 | 56.93 |
| S13 | Experimented | 45.52 | 45.38 | 36.54 | 36.71 | 38.95 | 36.72 |
|  | ANN Predicted | 45.01 | 45.54 | 36.59 | 36.73 | 39.13 | 36.55 |

The error metrics further confirm the robustness of the developed ANN model, as evidenced by low Root Mean Square Error (RMSE = 0.438), Mean Absolute Error (MAE = 0.287), and minimal bias (−0.098). These results indicate high prediction accuracy, low dispersion of errors, and negligible systematic overestimation or underestimation.

The regression performance plot for Sample 1 is illustrated in Fig. 3, demonstrating the close clustering of predicted values along the 1:1 line. The regression coefficient (R) values for all samples are provided in Table 6, representing the strength of correlation between experimentally observed (actual) and ANN-predicted WQI values. The consistently high R values across samples confirm the reliability and generalization capability of the developed ANN model.

Figure 3 Regression performance of the ANN model during training, validation, and testing phases (Sample S1).

Table 6:   Regression coefficient (R) values of the samples

| Sample ID | Training | validation | Testing | All |
| --- | --- | --- | --- | --- |
| S1 | 0.99997 | 0.99633 | 0.97176 | 0.99509 |
| S2 | 0.97815 | 0.99412 | 0.98929 | 0.97985 |
| S3 | 0.99982 | 0.99643 | 0.99301 | 0.99575 |
| S4 | 0.96993 | 1 | 0.92065 | 0.97223 |
| S5 | 0.99956 | 0.98976 | 0.98369 | 0.99314 |
| S6 | 0.94363 | 0.99709 | 0.99954 | 0.97622 |
| S7 | 0.99984 | 0.99652 | 0.99014 | 0.99761 |
| S8 | 0.97688 | 0.98551 | 0.98003 | 0.97487 |
| S9 | 0.99783 | 0.96785 | 0.96241 | 0.98251 |
| S10 | 0.98274 | 0.99831 | 0.97242 | 0.98216 |
| S11 | 0.98124 | 0.99023 | 0.98488 | 0.98363 |
| S12 | 0.99489 | 0.99727 | 0.97409 | 0.99229 |
| S13 | 0.99964 | 0.99927 | 0.99449 | 0.99782 |

.

# CONCLUSIONS

(i) Based on WA-WQI analysis from Table 4,

a) The results indicated that the water samples collected from ten sampling stations (S1, S2, S3, S4, S5, S6, S7, S8, S10 and, S13) of the study area meet the required standards for drinking purpose as their status was classified under good category.

b) The remaining water samples at three sampling stations (S9, S11, S12) Mittivalasa, Parannavalasa and Puroithunivalasa reported as Poor category. It is proposed to take appropriate treatment measures before the consumption of water collected from these stations.

(ii) Based on ANN Performance Evaluation from Table 4 and Table 5,

It is observed that the 15–5–1 ANN developed model achieved strong predictive performance with consistent generalization across independent datasets. The framework enables rapid WQI estimation from routine hydro chemical measurements and may serve as a practical decision-support tool in hard-rock aquifer systems.

Future work should explore reduced-parameter ANN models and cross-regional validation to enhance scalability.

Limitations and Future Research

This study is limited by its reliance on physicochemical measurements from 13 stations within a single hard-rock aquifer setting; therefore, the reported WQI patterns and model performance may not directly generalize to other hydrogeological domains or to locations influenced by different land-use and contamination sources. In addition, the ANN was trained to reproduce WQI derived from the same input parameters, which may inflate predictive metrics and does not demonstrate true forecasting of future water quality conditions. Quality-assurance information (e.g., duplicates, blanks, detection limits, ionic balance error) was not explicitly assessed, and seasonal/monsoon variability was not examined in detail.

Future research should incorporate multi-year datasets from diverse aquifers, evaluate time-aware validation (e.g., temporal holdout), and explore models that predict future WQI or key parameters using hydroclimatic drivers. Interpretable methods (e.g., SHAP/feature sensitivity) and uncertainty quantification should be added to identify dominant controls and improve decision support for groundwater management.

Acknowledgements: The authors acknowledge B. Sravya Research Scholar, IIT Bombay for technical support and G. Gowri Shankar, AE of APRWS & S department for his support in sample collection.

Competing interests

The authors declare no conflict of interest

Authors’ contributions

Author A’ designed the study, performed the statistical analysis, wrote the protocol, and wrote the first draft of the manuscript. ‘Author B’ and ‘Author C’ managed the analyses of the study. ‘Author C’ managed the literature searches. All authors read and approved the final manuscript. All authors contributed to the conception, analysis, and writing of the manuscript and approved the final version.

Funding: This research did not receive any specific grant from funding agencies in the public, commercial, or not-for-profit sectors.

Data Availability: The data supporting the findings of this study are available from the corresponding author upon reasonable request.

Disclaimer (Artificial Intelligence)

Author(s) hereby declares that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc) and   text-to-image generators have been used during writing or editing of this manuscript.

5. REFERENCES

APHA, AWWA, WEF, (2017). Standard Methods for the Examination of Water and Wastewater, 23rd ed. American Public Health Association, Washington, DC.

Appala Raju, N., Harikrishna, K., Satyanarayana, P., Suneetha, P., Sachi Devi, S., (2014). Spatial distribution of groundwater quality in Vizianagaram District, Andhra Pradesh, India. International Journal of Science, Environment and Technology 3 (1), 148–160.

BIS, (2012). Indian Standard Drinking Water Specification IS 10500, Bureau of Indian Standards, New Delhi

Basim, H.K., Jasimb, M.M., Alsaqqar, A.S., (2018). Artificial neural network model for the prediction of groundwater quality. Civil Engineering Journal 4 (12), 2959–2970.

Brown, R.M., McClelland, N.I., Deininger, R.A., O’Connor, M.F., (1972). A water quality index – crashing the psychological barrier. In: Indicators of Environmental Quality. Springer, Boston, pp. 173–182. https://doi.org/10.1007/978-1-4684-1698-5_15

Chen, H., Zhao, Q., Yaseen, Z.M., (2025). Explainable artificial intelligence for water quality assessment and risk analysis.Water Research, 225, 119362.

García-del-Toro, E.M., Mas-López, M.I., Mateo, L.F., Quijano, M.Á., (2025). Groundwater quality prediction for drinking and irrigation uses using artificial neural networks. Applied Water Science 15, 247.

Ghadai, M., Satapathy, D.P., Krishnasamy, S., Ramalingam, M., Sreelal, G.P., Dhilipkumar, B., (2022). Artificial neural network and weighted arithmetic indexing approach for surface water quality assessment of the Brahmani River. Global NEST Journal 24 (4), 562–568.

Gummadi, S., Swarnalatha, G., Venkataratnamma, V., Vishnuvardhan, Z., (2014). Determination of water quality index for groundwater of Bapatla Mandal, Guntur District, Andhra Pradesh, India. International Journal of Engineering Research & Technology 3 (3), 77–80.

Gao.Y, H. Qian, W. Ren, H. Wang, F. Liu, F. Yang (2020) Hydrogeochemical characterization and quality assessment of groundwater based on integrated-weight water quality index in a concentrated urban area J. Clean. Prod., 260, p. 121006.

Liu, L., Sun, W., Zhao, X., (2022). Scenario-based prediction of groundwater quality under climate change using machine learning.Science of the Total Environment, 812, 152467.

Li, J., Wang, Y., Zhang, X., et al., (2023). Deep learning models for groundwater quality prediction: a comparative analysis.Journal of Hydrology, 621, 127⁠—139.

Kumar, Ravishankar, R, S. Mittal, P.K. Sahoo, S.K. Sahoo (2021) Source apportionment, chemometric pattern recognition and health risk assessment of groundwater from southwestern Punjab, India Environ. Geochem. Health, Vol. 43, 733-755.

K.Krishna Kumar (2012) Groundwater- Fluoride: Geochemical Assessment along Bay of Bengal of Vizianagaram. Journal of Indian Water Works Association 125-133.

Maier, H.R., Dandy, G.C., (2000). Neural networks for the prediction and forecasting of water resources variables: A review of modelling issues and applications. Environmental Modelling & Software 15, 101–124.

Muthukumaran, V., Vinoth Kumar, V.,(2022). Water quality index process using artificial neural networks. International Journal of Information Technology, Research and Applications 1 (1), 33–37. https://doi.org/10.59461/ijitra.v1i1.12

Nayar, R., (2020). Assessment of water quality index and monitoring of pollutants by physicochemical analysis in water bodies: A review. International Journal of Engineering Research & Technology 9 (1), 178–185.

Pavan Kumar, G.V.S.R., Chandrasekhar, T., SreeramaMurty, B., (2017a). Assessment of groundwater quality status based on water quality index method in Konada and Chintapalli villages of Vizianagaram District, Andhra Pradesh, India. Journal of the Indian Chemical Society 94, 1–9.

Pavan Kumar, G.V.S.R., Partha Sarathi, T.V.N., Srinivasa Rao, K., (2017b). Assessment and prediction of water quality index using artificial neural networks in selected coastal villages of Vizianagaram District, Andhra Pradesh. Journal of the Indian Chemical Society 94, 1–8.

Rabah Ismail, Rawashdeh, A., Al-Mattarneh, H., Hatamleh, R., Telfah, D.B., Jaradat, A.,(2024). Artificial intelligence for application in water engineering: The use of ANN to determine water quality index in rivers. Civil Engineering Journal 10 (7), 2261–2274.

Ramakrishnaiah, C.R., Sadashivaiah, C., Ranganna, G., (2009). Assessment of water quality index for the groundwater in Tumkur Taluk, Karnataka State, India. E-Journal of Chemistry 6 (2), 523–530.

Rajankar, P. N, S. R. Gulhane, D. H. Tambekar,   D. S. Ramteke and S. R. Wate (2009) Water Quality Assessment of Groundwater Resources in Nagpur Region (India) Based on WQI. E-Journal of Chemistry 2009, 6(3), 905-908.

Subba Rao N, B. Sunitha, N. Adimalla, M. Chaudhary (2020). Quality criteria for groundwater use from a rural part of Wanaparthy District, Telangana State, India, through ionic spatial distribution (ISD), entropy water quality index (EWQI) and principal component analysis (PCA)

Environ. Geochem. Health, 42 (2), pp. 579-599

Swarna Latha. P, Nageswara Rao. K (2010).Assessment and spatial distribution of quality of Groundwater in zone-II and III, Greater Visakhapatnam, India using Water Quality Index and GIS. International Journal of Environmental Sciences, Vol. 1 (2), 198-212

Srinivasa Rao, G., Nageswararao, G., (2013). Assessment of groundwater quality using water quality index. Archives of Environmental Sciences 7, 1–5.

Singh, R.K., Gupta, P.K., Maity, S., (2024). Hybrid machine learning and geostatistical framework for groundwater quality prediction.Environmental Modelling & Software, 169, 105271.

Vasanthavigar, M., Srinivasamoorthy, K., Vijayaragavan, K., Rajiv Ganthi, R., Chidambaram, S., Anandan, P., Manivannan, R., Vasudevan, S., (2010). Application of water quality index for groundwater quality assessment: Thirumanimuttar sub-basin, Tamil Nadu, India. Environmental Monitoring and Assessment 171, 595–609.

WHO, (2017). Guidelines for Drinking-Water Quality, 4th edn, World Health Organization, Geneva.

Wu, H. Zhou, S. He, Y. Zhang Comprehensive understanding of groundwater quality for domestic and agricultural purposes in terms of health risks in a coal mine area of the Ordos basin, north of the Chinese Loess Plateau

Environ Earth Sci, 78 (15) (2019), p. 446.

Sajib, A.M., Diganta, M.T.M., Rahman, A., Dabrowski, T., Olbert, A.I., Uddin, M.G. (2023). Developing a novel tool for assessing the groundwater incorporating water quality index and machine learning approach. Groundwater for Sustainable Development, 101049. https://doi.org/10.1016/j.gsd.2023.101049

Ahn, S.H., Jeong, D.H., Kim, M., Lee, T.K., Kim, H.-K. (2023). Prediction of groundwater quality index to assess suitability for drinking purpose using averaged neural network and geospatial analysis. Ecotoxicology and Environmental Safety, 265, 115485. https://doi.org/10.1016/j.ecoenv.2023.115485

Maiti, S., Gupta, S. & Gupta, P.K. Prediction of Groundwater Quality Index and Identification of Key Variables Using Bayesian Neural Network. Water Air Soil Pollut 235, 664 (2024). https://doi.org/10.1007/s11270-024-07459-w

Ehteram, M., Ahmed, A. N., Sherif, M., & El-Shafie, A. (2024). An advanced deep learning model for predicting water quality index. Ecological Indicators, 160, 111806. https://doi.org/10.1016/j.ecolind.2024.111806

Haggerty, R., Sun, J., Yu, H., & Li, Y. (2023). Application of machine learning in groundwater quality modeling-A comprehensive review. Water Research, 233, 119745. https://doi.org/10.1016/j.watres.2023.119745

Basavaraja, D., Kambale, J. B., Jahnavi, A. P., & Hadimani, D. K. (2025). Evaluation of Groundwater Quality in Northeastern Region of Karnataka, India Using Correlation and Regression Analysis. Archives of Current Research International, 25(11), 41–49. https://doi.org/10.9734/acri/2025/v25i111599