HUMAN HEALTH RISK ASSESSMENT ASSOCIATED WITH THE INHALATION OF AIR POLLUTANTS AROUND INDUSTRIAL AREA IN BONNY ISLAND, RIVERS STATE

Abstract

Burning fossil fuels for industrial, commercial, and transportation purposes directly contributes to air pollution. Extended long-term exposure to the pollutants, which harm the neurological, reproductive and respiratory systems and cause cancer and, in rare cases, deaths, can exacerbate these issues. This study assessed the environmental and health risks associated with criteria pollutants in the air of Bonny Island. For this study Samples were collected from twenty sampling points using a digital gas monitor. Air samples were collected for the dry and wet seasons. Samples were collected during the time of active release of the pollutants. This research employed the quantitative AP-HRA in the risk assessment. The risk due to inhalation of air pollutants in the ambient air was estimated for CO, NOx, O3, SO2, PM2.5, and PM10 using specific models for respective risks. The risks for the exposure quantification of CO, NOx, O3, SO2, PM2.5, and PM10 for inhalation of air pollution were far below the annual threshold of 0.004, 10, 60, 40, 5 and 15, respectively, as recommended by the World Health Organisation (WHO). In addition, the calculated risk of inhalation of CO, NOx, O3, SO2, PM2.5, and PM10 was below the USEPA acceptable risk in the study location for the 20 days during the wet seasons. It is recommended that Further research should be carried out on the effects of air dispersion on particulate matter and gaseous pollutants.

Keywords: human health, ambient air, air pollutant, risk assessment, industrial activities.

- Introduction

Air pollution is an important determinant of health (WHO, 2014). The study by Ede et al (2011) and Ukpere et al (2018) examined the characteristics of Bonny, Nigeria, and air quality that is often attributed to LNG installations. Bonny-like industrial districts have revealed that these places typically have much higher amounts of these air pollutants, such as Particulate matter (PM2.5 and PM10), sulfur dioxide (SO2), nitrogen oxides (NOx), carbon monoxide (CO), volatile organic compounds (VOCs), ozone (O3), and other trace gases than non-industrial or residential areas. Consequently, the negative health effects of air pollution disproportionately affect the populations that live in these places (Esievo et al., 2024).

They claimed that the plant complex produces air pollutants that are spread by weather-related factors. Considering the amount of investment on the island, Bonny is emerging as possibly the most significant industrial enclave in Nigeria (Finebone,2022; Watts, 2024). It serves as the country’s main centre for the export of both crude oil and liquefied natural gas (Akintoye et al., 2024; Jumbo & Ihuah, 2024). Bonny is a community with lots of human settlements, with environmental changes that have had a significant impact. Ukpere et al (2018) argue that burning fossil fuels for industrial, commercial, and transportation purposes directly contributes to air pollution.

Combustion sources for energy and power generation that are used for dehydration and liquefaction processes result in air emissions (continuous or intermittent) from LNG facilities (Minet et al., 2025). Other sources are electric gas turbines, boilers, incinerators, oxidisers, and other equipment in the natural gas processing chain located at the Bonny LNG plant. In addition to fugitive sources, flaring and venting also produce emissions. NOx, CO, and, in the case of sour gases, SO2, are the principal gases from which sources of PM are also released during most combustion processes (Ede et al 2011).

It has been documented that those human ventures in commercial cities; like those from miniature and medium sized industries namely dyes, fertilizer, ceramics, glass and textile industries, to sales of electronics and household items, abattoir services, auto repairs, hotel and restaurants, shopping malls and freelance street side trading have a significant impact on air standard (Kposoruchi & Dumka, 2022). Pachoulis et al (2022), Amiriheaobu et al (2020), and Kim et al (2014) estimated the health risks resulting from short-term exposure to an aggravated environment in an industrial area, in a highly populated environment. The risk estimation was performed using different approaches suggested in relative literatures. Part of the study was to monitor the air quality in an industrial zone of Africa, was including 24h measurement of PM2.5.

Sridevi et al (2019) used the health risk assessment model developed by the USEPA for the quantification of 10 PM2.5-bound trace elements by using ICP-OES, source apportionment by using principal component analysis and health risk posed by PM2.5-bound trace elements. In their studies, Harrison et al (2017) estimated the health risk for particulate matter in their research on health risk associated with airborne particulate matter and its components in Jeddah, Saudi Arabia. Many scholars have carried out several studies in human health risk assessment.

Li et al (2016), Jackson (2005), and Lim et al (2010) conducted research on the characteristics, sources and health risk assessment of toxic heavy metals in PM2.5 at a megacity of southwest China. In their research paper, Prakash et al (2018) investigated the chemical characterisation and quantitative assessment of source-specific health risk of trace metals in PM10 at a road site of Delhi, India. In their studies, Emmanouil et al (2016) researched particulate matter, air pollution, source origin and human health risk based on PM10 metal content in Volos city, Greece. Huang et al (2014) carried out studies on contamination and risk assessment based on bio accessibility via ingestion and inhalation of metalloid in outdoor and indoor particles from urban centres of Guangzhou, China.

High levels of air pollution cause disease symptoms and states of varying severity in those exposed (Manisalidis et al 2020). These consequences are divided into those that affect health in the short and long term. A recent epidemiological study from Harvard School of Public Health found that exposure errors and different epidemiological methodologies have made it difficult to determine the exact magnitudes of the short and long-term effects.

For more effective evaluation of short- and long-term human exposure data, new models are proposed (Kloog et al, 2013). As a result, they include both general concerns for both types of effects in this section as well as the more frequent short-and long-term health effects, as these effects are frequently influenced by the environment, dose, and individual sensitivity. The short-term symptoms are transient and can range from minor discomfort, Such as irritation of the eyes, nose, skin and throat, to more serious conditions, such as asthma, pneumonia, bronchitis and lung and heart issues. Headaches, nausea and light-headedness can also result from brief exposure to air pollution (Singh et al., 2025). Extended long-term exposure to the pollutants, which harm the neurological, reproductive and respiratory systems and cause cancer and, in rare cases, death, can exacerbate these issues (Manisalidis et al 2020).

The respiratory system is seriously harmed by PMs, dust, benzene and O3 (Kurrt et al, 2016). Additionally, there is an added danger if a respiratory condition like asthma is already present (Guarneri & Balms, 2014). After an acute exposure to contaminants, vocal changes may be noted if the trachea is affected. Air pollution may cause chronic obstructive pulmonary disease (COPD), which raises morbidity and mortality (Jiang et al, 2016). The main risk factors for COPD include the long-term consequences of traffic, industrial air pollution and fuel combustion, according to (Jiang et al, 2016).

After exposure to air pollution, numerous cardiovascular consequences have been noted (Guurdrel et al, 2017). Long-term exposure to traffic emissions has been linked to coronary arteriosclerosis (Hoftmann et al 2007), whilst short-term exposure has been linked to hypertension, stroke, myocardial infarction, and heart failure. According to studies conducted on humans, ventricular hypertrophy can develop with prolonged exposure to nitrogen dioxide (NO2) (Katholi & Couri, 2011). Due to the impacts of exposure to air pollution, especially health impacts, as revealed by many scholars, it becomes imperative for and exposure assessment as well as air pollution health risk assessment (AP-HRA). AP- HRA estimates the health impact to be expected, measures that affect air quality, in different socio-economic, environmental and policy circumstances.

An AP-HRA may be qualitative or quantitative; it generally assesses

- The amount of air pollution

Present, i.e., pollution concentrations. (ii) the amount of contact (exposure) of the targeted population, and (iii) how harmful the concentration is for human health, i.e the resulting risks to the exposed (US-EPA 1989).

2. Materials and Methods

2.1. Description of the study Area.

The study area is the Bonny local government area in Rivers State, with the heavy presence of multinational companies’ operations. Bonny Island lies in a part of 40 kilometres south of Port Harcourt (Fig 1) with latitudes 40 52’N and 50 02’N, and longitudes 60 56’E and 70 04’E, with an estimated population of 30,200 people (population Density, 2022). The Island has a rather flat topography, with an elevation of 3.05 atmospheric mean sea level and a total land area of 214.52m2, with tidal floods and land subsidence affecting 70% of its size (NLNG, 2005). The Island is directly connected to the Atlantic Ocean, where giant oil tankers export crude oil. The rainfall in the Bonny Island ranges from 2301mm to 3670mm during the wet season (March to November) and 43 to 97mm during the dry season (December to March) (Adejunon, 2012).

Figure 1: Study Area in Bonny Local Government Area of Rivers State

2.2 Sampling, Design and Sample Collection.

The design has 20 sampling points with 3 replicates, consisting of 60 sampling points for air pollution.

These were carried out during the dry and wet seasons for two different months, respectively. The samples were collected at a distance of 100m from the discharging point.

2.2.1 Sample Collection

Samples were collected from twenty sampling points using a digital gas monitor, which gives an instantaneous reading/recording of data. The digital gas monitor is a high-volume sampler (Environed HVS APM – 410/451). The sampler has attachment APM-411 for gas analysis. It is set at a height of 3 meters from the ground in the designated sampling points.

Air samples were collected for the dry and wet seasons. Samples were collected during the time of active release of the pollutants. Sampling was done during the day at regular intervals for a period of 8 hours according to ROO and WHO guidelines.

High-grade filter paper (GF/A) is used in the HVS for SPM. SOx and NOx were measured after the collected air was passed through the absorbent solutions. The absorbent solution is then analysed by a UV/VIS spectrophotometer. A portable gas monitor was used to measure some parameters.

2.3 Risk Assessment of Criteria Pollutants

2.3.1 Human Exposure Assessment

For inhalation of criteria pollutants, chronic daily intake (CDI) was calculated using equation 2.1 (24):

CDI (mgkg-day)=CAIRETEFEDBWAT							(2.1)

Where:

CDI = chronic daily intake (in mg · kg−1 · day−1)

CA = contaminant concentration in air (in mg · m−3)

AT = averaging time (in days, 70 years X 365 days/year = 25550 days/year)

ED = exposure duration (in years, 70 years)

EF = exposure frequency (in days · year−1, 365/year)

ET = exposure time (h · day−1 or h · event−1, 12 min = 0.2 hour/day)

IR = inhalation rate (in m3 · h−1, 8.33333 X 10-5)

BW = Body weight, Adult (kg, 70kg)

2.3.2 Risk Assessment

For the quantification of the risk in the study area, equation (2.2) was adopted according to (4).

Risk=Chronic daily intake(CDI)mgkg.day×cancerpotency factor CPFmgkg.day-1												 (2.2)

3. Results and Discussion

This research employed the quantitative AP-HRA in the risk assessment. Here, the pollutant concentrations were determined as shown in Tables 1 and 2. Then the amount of contact (exposure) of the targeted population was calculated as shown in Tables 3 and 4. Finally, the resulting risks to the exposed were calculated as shown in Tables 5 and 6.

3.1. The Amount of Air Pollution Presents I.E Pollution Concentrations

Table 1: Air Pollutants in Bonny Island during Dry Season

| Period, day | Parameters (µg/m3) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | CO | NOX | O3 | SO2 | PM2.5 | PM10 |
| Day 1 | 0.311 | 0.008 | 0.014 | 0.002 | 26.89 | 30.35 |
| Day 2 | 0.496 | 0.011 | 0.011 | 0.003 | 43.45 | 47.25 |
| Day 3 | 0.142 | 0.011 | 0.019 | 0.002 | 13.3 | 14.84 |
| Day 4 | 0.163 | 0.013 | 0.022 | 0.002 | 8.52 | 10.02 |
| Day 5 | 0.302 | 0.009 | 0.023 | 0.002 | 10.1 | 10.93 |
| Day 6 | 0.599 | 0.008 | 0.009 | 0.003 | 34 | 36.61 |
| Day 7 | 0.544 | 0.011 | 0.011 | 0.002 | 34.8 | 37.67 |
| Day 8 | 0.711 | 0.005 | 0.017 | 0.002 | 24.14 | 25.61 |
| Day 9 | 0.543 | 0.006 | 0.019 | 0.002 | 13.49 | 14.65 |
| Day 10 | 0.589 | 0.008 | 0.019 | 0.003 | 19.4 | 20.83 |
| Day 11 | 0.399 | 0.007 | 0.021 | 0.004 | 8.38 | 9.02 |
| Day 12 | 0.262 | 0.007 | 0.016 | 0.002 | 9.66 | 10.49 |
| Day 13 | 0.25 | 0.01 | 0.02 | 0.002 | 6.25 | 6.84 |
| Day 14 | 0.251 | 0.007 | 0.022 | 0.002 | 10.18 | 10.94 |
| Day 15 | 0.3 | 0.006 | 0.023 | 0.002 | 11.1 | 11.93 |
| Day 16 | 0.269 | 0.008 | 0.015 | 0.003 | 16.76 | 18.29 |
| Day 17 | 0.305 | 0.004 | 0.016 | 0.002 | 6.96 | 7.99 |
| Day 18 | 0.392 | 0.006 | 0.011 | 0.002 | 25.61 | 28.04 |
| Day 19 | 0.23 | 0.006 | 0.015 | 0.002 | 5.49 | 6.32 |
| Day 20 | 0.202 | 0.011 | 0.015 | 0.002 | 5.8 | 6.77 |
| WHO | 2 | 200 | 50 | 125 | 40 | 50 |

Table 2: Air Pollutants in Bonny Island during Wet Season

| Period, day | Parameters (µg/m3) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | CO | NOX | O3 | SO2 | PM2.5 | PM10 |
| Day 1 | 0.458 | 0.011 | 0.02 | 0.003 | 4.37 | 4.84 |
| Day 2 | 0.51 | 0.013 | 0.02 | 0.004 | 5.45 | 5.81 |
| Day 3 | 0.592 | 0.006 | 0.019 | 0.003 | 18.55 | 19.76 |
| Day 4 | 0.51 | 0.005 | 0.027 | 0.003 | 3.08 | 3.34 |
| Day 5 | 0.643 | 0.006 | 0.012 | 0.002 | 17.37 | 18.31 |
| Day 6 | 0.524 | 0.004 | 0.018 | 0.002 | 11.2 | 11.9 |
| Day 7 | 0.57 | 0.005 | 0.017 | 0.003 | 6.4 | 6.82 |
| Day 8 | 0.766 | 0.003 | 0.016 | 0.002 | 12.1 | 12.76 |
| Day 9 | 0.541 | 0.005 | 0.027 | 0.002 | 5 | 5.31 |
| Day 10 | 0.596 | 0.006 | 0.019 | 0.004 | 11.76 | 12.39 |
| Day 11 | 0.821 | 0.003 | 0.011 | 0.003 | 38.29 | 40.3 |
| Day 12 | 0.597 | 0.008 | 0.016 | 0.002 | 8.13 | 8.42 |
| Day 13 | 0.77 | 0.005 | 0.012 | 0.003 | 16.54 | 17.35 |
| Day 14 | 0.652 | 0.002 | 0.012 | 0.003 | 20.62 | 21.82 |
| Day 15 | 0.569 | 0.007 | 0.017 | 0.002 | 5.9 | 6.18 |
| Day 16 | 0.523 | 0.003 | 0.024 | 0.002 | 5.84 | 6.13 |
| Day 17 | 0.52 | 0.002 | 0.017 | 0.002 | 9.6 | 10.07 |
| Day 18 | 0.531 | 0.003 | 0.02 | 0.002 | 9.21 | 9.76 |
| Day 19 | 0.758 | 0.004 | 0.018 | 0.004 | 11.86 | 12.32 |
| Day 20 | 0.599 | 0.005 | 0.027 | 0.002 | 11.19 | 11.49 |
| WHO | 2 | 200 | 50 | 125 | 40 | 50 |

Table 1 and 2 above shows the concentrations of the criteria pollutions including (CO, NOx, O3-, SO2, PM2.5, and PM10).

3.2 Human exposure assessment

3.21 Carbon Monoxide (CO).

Fig. 2 shows the inhalation exposure of CO in the sampling location of the Bonny Island environment during the dry and wet seasons. This CO exposure assessment is also shown in Tables 3 and 4. These assessments were to indicate the level of exposure through inhalation of the breathing air. The levels of CO in the air were far below the 0.004Hg/m3 annual threshold as recommended by the World Health Organisation (1). The results have shown that the levels of CO exposure to human health in this study, as a known pollutant, will not result in illness.

Figure 2: CO Inhalation Exposure Assessment in Bonny Environment with Per cent Error Bar

3.2.2. Nitrogen Oxide (NOx)

Fig. 3 shows the inhalation exposure of NOx in the sampling location of Bonny Island environment during the dry and wet seasons. This NOx exposure assessment is also shown in Tables 3 and 4. These assessments were to indicate the level of exposure through inhalation of the breathing air. The levels of NOx in the air were far below the 10Hg/m3 annual threshold as recommended by the World Health Organisation (1). The results have shown that the levels of NOx exposure to humans in this study, as a known pollutant, will not result in illness.

Figure 3: NOx Inhalation Exposure Assessment in Bonny Environment with Per cent Error Bar

3.2.3 Ozone (03)

Fig. 4 shows the inhalation exposure of 03 in the sampling location of Bonny Island environment during the dry and wet seasons. This 03 exposure assessment is also shown in Tables 3 and 4. These assessments were to indicate the level of exposure through inhalation of the breathing air. The levels of 03 in the air were far below the 60 Hg/m3 annual threshold as recommended by the World Health Organisation (1). The results have shown that the levels of 03 exposure to humans in this study, as a known pollutant, will not result in illness.

Figure 4: O3 Inhalation Exposure Assessment in Bonny Environment with Per cent Error Bar

3.2.4 Sulphur Dioxide (SO2)

Fig. 5 shows the inhalation exposure of SO2 in the sampling location of Bonny Island environment during the dry and wet seasons. This SO2 exposure assessment is also shown in Tables 3 and 4. These assessments were to indicate the level of exposure through inhalation of the breathing air. The levels of SO2 in the air were far below the 40Hg/m3 annual threshold as recommended by the World Health Organisation (1). The results have shown that the levels of SO2 exposure to humans in this study, as a known pollutant, will not result in illness.

Figure 5: SO2 Inhalation Exposure Assessment in Bonny Environment with Per cent Error Bar

3.2.5 Particulate matter ≤ 2.5mm (PM2.5)

Fig. 6 shows the inhalation exposure of PM2.5 in the sampling location of Bonny Island environment during the dry and wet seasons. This PM2.5 exposure assessment is also shown in Tables 3 and 4. These assessments were to indicate the level of exposure through inhalation of the breathing air. The levels of PM2.5 in the air were far below the 5Hg/m3 annual threshold as recommended by the World Health Organisation (1). The results have shown that the levels of PM2.5 exposure to humans in this study, as a known pollutant, will not result in illness.

Figure 6: PM2.5 Inhalation Exposure Assessment in Bonny Environment with Per cent Error Bar

3.2.6 Particulate matter ≤ 10mm (PM10)

Fig. 7 shows the inhalation exposure of PM10 in the sampling location of Bonny Island environment during the dry and wet seasons. This PM10 exposure assessment is also shown in Tables 3 and 4. These assessments were to indicate the level of exposure through inhalation of the breathing air. The levels of PM10 in the air were far below the 15Hg/m3 annual threshold as recommended by the World Health Organisation (1). The results have shown that the levels of PM10 exposure to humans in this study, as a known pollutant, will not result in illness.

Figure 7: PM10 Inhalation Exposure Assessment in Bonny Environment with Per cent Error Bar

Table 3: Inhalation of Air Pollutants in Bonny Island during Dry Season

| Period, day | Parameters (µg/m3) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | CO | NOX | O3 | SO2 | PM2.5 | PM10 |
| Day 1 | 7.40E-08 | 1.90E-09 | 3.33E-09 | 4.76E-10 | 6.40E-06 | 7.23E-06 |
| Day 2 | 1.18E-07 | 2.62E-09 | 2.62E-09 | 7.14E-10 | 1.03E-05 | 1.12E-05 |
| Day 3 | 3.38E-08 | 2.62E-09 | 4.52E-09 | 4.76E-10 | 3.17E-06 | 3.53E-06 |
| Day 4 | 3.88E-08 | 3.10E-09 | 5.24E-09 | 4.76E-10 | 2.03E-06 | 2.39E-06 |
| Day 5 | 7.19E-08 | 2.14E-09 | 5.48E-09 | 4.76E-10 | 2.40E-06 | 2.60E-06 |
| Day 6 | 1.43E-07 | 1.90E-09 | 2.14E-09 | 7.14E-10 | 8.10E-06 | 8.72E-06 |
| Day 7 | 1.30E-07 | 2.62E-09 | 2.62E-09 | 4.76E-10 | 8.29E-06 | 8.97E-06 |
| Day 8 | 1.69E-07 | 1.19E-09 | 4.05E-09 | 4.76E-10 | 5.75E-06 | 6.10E-06 |
| Day 9 | 1.29E-07 | 1.43E-09 | 4.52E-09 | 4.76E-10 | 3.21E-06 | 3.49E-06 |
| Day 10 | 1.40E-07 | 1.90E-09 | 4.52E-09 | 7.14E-10 | 4.62E-06 | 4.96E-06 |
| Day 11 | 9.50E-08 | 1.67E-09 | 5.00E-09 | 9.52E-10 | 2.00E-06 | 2.15E-06 |
| Day 12 | 6.24E-08 | 1.67E-09 | 3.81E-09 | 4.76E-10 | 2.30E-06 | 2.50E-06 |
| Day 13 | 5.95E-08 | 2.38E-09 | 4.76E-09 | 4.76E-10 | 1.49E-06 | 1.63E-06 |
| Day 14 | 5.98E-08 | 1.67E-09 | 5.24E-09 | 4.76E-10 | 2.42E-06 | 2.60E-06 |
| Day 15 | 7.14E-08 | 1.43E-09 | 5.48E-09 | 4.76E-10 | 2.64E-06 | 2.84E-06 |
| Day 16 | 6.40E-08 | 1.90E-09 | 3.57E-09 | 7.14E-10 | 3.99E-06 | 4.35E-06 |
| Day 17 | 7.26E-08 | 9.52E-10 | 3.81E-09 | 4.76E-10 | 1.66E-06 | 1.90E-06 |
| Day 18 | 9.33E-08 | 1.43E-09 | 2.62E-09 | 4.76E-10 | 6.10E-06 | 6.68E-06 |
| Day 19 | 5.48E-08 | 1.43E-09 | 3.57E-09 | 4.76E-10 | 1.31E-06 | 1.50E-06 |
| Day 20 | 4.81E-08 | 2.62E-09 | 3.57E-09 | 4.76E-10 | 1.38E-06 | 1.61E-06 |
| WHO | 0.004 | 10 | 60 | 40 | 5 | 15 |

Table 4: Air Pollutants Inhalation in Bonny Island during the Wet Season

| Period, day | Parameters (µg/m3) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | CO | NOX | O3 | SO2 | PM2.5 | PM10 |
| Day 1 | 1.09E-07 | 2.62E-09 | 4.76E-09 | 7.14E-10 | 1.04E-06 | 1.15E-06 |
| Day 2 | 1.21E-07 | 3.10E-09 | 4.76E-09 | 9.52E-10 | 1.30E-06 | 1.38E-06 |
| Day 3 | 1.41E-07 | 1.43E-09 | 4.52E-09 | 7.14E-10 | 4.42E-06 | 4.70E-06 |
| Day 4 | 1.21E-07 | 1.19E-09 | 6.43E-09 | 7.14E-10 | 7.33E-07 | 7.95E-07 |
| Day 5 | 1.53E-07 | 1.43E-09 | 2.86E-09 | 4.76E-10 | 4.14E-06 | 4.36E-06 |
| Day 6 | 1.25E-07 | 9.52E-10 | 4.29E-09 | 4.76E-10 | 2.67E-06 | 2.83E-06 |
| Day 7 | 1.36E-07 | 1.19E-09 | 4.05E-09 | 7.14E-10 | 1.52E-06 | 1.62E-06 |
| Day 8 | 1.82E-07 | 7.14E-10 | 3.81E-09 | 4.76E-10 | 2.88E-06 | 3.04E-06 |
| Day 9 | 1.29E-07 | 1.19E-09 | 6.43E-09 | 4.76E-10 | 1.19E-06 | 1.26E-06 |
| Day 10 | 1.42E-07 | 1.43E-09 | 4.53E-09 | 9.52E-10 | 2.80E-06 | 2.95E-06 |
| Day 11 | 1.95E-07 | 7.14E-10 | 2.62E-09 | 7.14E-10 | 9.12E-06 | 9.60E-06 |
| Day 12 | 1.42E-07 | 1.90E-09 | 3.81E-09 | 4.76E-10 | 1.94E-06 | 2.00E-06 |
| Day 13 | 1.83E-07 | 1.19E-09 | 2.86E-09 | 7.14E-10 | 3.94E-06 | 4.13E-06 |
| Day 14 | 1.55E-07 | 4.76E-10 | 2.86E-09 | 7.14E-10 | 4.91E-06 | 5.20E-06 |
| Day 15 | 1.35E-07 | 1.67E-09 | 4.05E-09 | 4.76E-10 | 1.40E-06 | 1.47E-06 |
| Day 16 | 1.25E-07 | 7.14E-10 | 5.71E-09 | 4.76E-10 | 1.39E-06 | 1.46E-06 |
| Day 17 | 1.24E-07 | 4.76E-10 | 4.05E-09 | 4.76E-10 | 2.29E-06 | 2.40E-06 |
| Day 18 | 1.26E-07 | 7.14E-10 | 4.76E-09 | 4.76E-10 | 2.19E-06 | 2.32E-06 |
| Day 19 | 1.80E-07 | 9.52E-10 | 4.29E-09 | 9.52E-10 | 2.82E-06 | 2.93E-06 |
| Day 20 | 1.43E-07 | 1.19E-09 | 6.43E-09 | 4.76E-10 | 2.66E-06 | 2.74E-06 |
| WHO | 0.004 | 10 | 60 | 40 | 5 | 15 |

3.3 Risk Assessment

3.3.1. Carbon Monoxide (Co)

Fig 8 shows the calculated risk of CO substance inhalation in breathing Bonny Island air during the dry and wet seasons. The CO air pollutant risk calculated is probably less harmful to humans in the studied area (Tables 5 and 6). CO was detected on all sampling days in the studied location during the experimental periods (dry and wet seasons). The calculated daily inhalations of CO were below the USEPA acceptable risk of 1 x 10-6 in all the studied days of the sampling location during dry and wet seasons. The results of the risk calculated for humans during both dry and wet seasons indicate that humans using this air for inhalation may not be prone to lung diseases.

Figure 8: Risk of Inhalation of CO in Bonny Atmosphere

3.3.2. Nitrogen Oxide (NOx).

The calculated risk from inhaling NOx chemicals while breathing in the Bonny Island environment is shown in Fig 9 for both the dry and wet seasons. No risks of being probably harmful have been assessed and are listed in Tables 5 and 6. NOx was found on every sampling day in the sampling position of the location.

NOx calculated daily intakes while breathing in air from the environment were significantly lower than the USEPA tolerable risk of 1.0 x 10-6 in all the days of the sampling area during dry and wet seasons. This suggests that people who use these environs aim for breathing may not be at risk of developing illness through inhalation while breathing air in the Bonny Island environment.

Figure 9: Risk of Inhalation of NOx in Bonny Atmosphere

3.3.3 Ozone (O3)

The calculated risk from inhaling O3 chemicals while breathing in the Bonny Island environment is shown in Fig 10 for both the dry and wet seasons. No risks of being probably harmful have been assessed and are listed in Tables 5 and 6. O3 was found on every sampling day in the sampling position of the location.

O3 calculated daily intakes while breathing in air from the environment were significantly lower than the USEPA tolerable risk of 1.0 x 10-6 in all the days of the sampling area during dry and wet seasons. This suggests that people who use these environs aim for breathing may not be at risk of developing illness through inhalation while breathing air in the Bonny Island environment.

Figure 10: Risk of Inhalation of O3 in Bonny Atmosphere

3.3.4 Sulphur Dioxide (SO2)

The calculated risk from inhaling SO2 chemicals while breathing in the Bonny Island environment is shown in Fig 11 for both the dry and wet seasons. No risks of being probably harmful have been assessed and are listed in Tables 5 and 6. SO2 was found on every sampling day in the sampling position of the location.

SO2 calculated daily intakes while breathing in air from the environment were significantly lower than the USEPA tolerable risk of 1.0 x 10-6 in all the days of the sampling area during dry and wet seasons. This suggests that people who use these environs aim for breathing may not be at risk of developing illness through inhalation while breathing air in the Bonny Island environment.

Figure 11: Risk of Inhalation of SO2 in Bonny Atmosphere

3.3.5 Particulate Matter ≤  2.5Ʋm (PM2.5)

The calculated risk from inhaling PM2.5 chemicals while breathing in the Bonny Island environment is shown in Fig 12 for both the dry and wet seasons. No risks of being probably harmful have been assessed and are listed in Tables 5 and 6. PM2.5 was found on every sampling day in the sampling position of the location.

PM2.5 calculated daily intakes while breathing in air from the environment were significantly lower than the USEPA tolerable risk of 1.0 x 10-6 in all the days of the sampling area during dry and wet seasons. This suggests that people who use the air for breathing may not be at risk of developing illness through inhalation while breathing air in the Bonny Island environment.

Figure 12: Risk of Inhalation of PM2.5 in Bonny Atmosphere

3.3.6 Particulate Matter ≤ 10Ʋm (PM10)

The calculated risk from inhaling PM10 chemicals while breathing in the Bonny Island environment is shown in Fig 13 for both the dry and wet seasons. No risks of being probably harmful have been assessed and are listed in Tables 5 and 6. PM10 was found on every sampling day in the sampling position of the location.

PM10 calculated daily intakes while breathing in air from the environment were significantly lower than the USEPA tolerable risk of 1.0 x 10-6 in all the days of the sampling area during dry and wet seasons. This suggests that people who use these environs aim for breathing may not be at risk of developing illness through inhalation while breathing air in the Bonny Island environment.

Figure 13: Risk of Inhalation of PM10 in Bonny Atmosphere

Table 5: Risk of Inhalation of Air Pollutants in Bonny Island during Dry Season

| Period, day | Parameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | CO | NOX | O3 | SO2 | PM2.5 | PM10 |
| Day 1 | 7.40E-14 | 1.90E-15 | 3.33E-15 | 4.76E-16 | 6.40E-12 | 7.23E-12 |
| Day 2 | 1.18E-13 | 2.62E-15 | 2.62E-15 | 7.14E-16 | 1.03E-11 | 1.12E-11 |
| Day 3 | 3.38E-14 | 2.62E-15 | 4.52E-15 | 4.76E-16 | 3.17E-12 | 3.53E-12 |
| Day 4 | 3.88E-14 | 3.10E-15 | 5.24E-15 | 4.76E-16 | 2.03E-12 | 2.39E-12 |
| Day 5 | 7.19E-14 | 2.14E-15 | 5.48E-15 | 4.76E-16 | 2.40E-12 | 2.60E-12 |
| Day 6 | 1.43E-13 | 1.90E-15 | 2.14E-15 | 7.14E-16 | 8.10E-12 | 8.72E-12 |
| Day 7 | 1.30E-13 | 2.62E-15 | 2.62E-15 | 4.76E-16 | 8.29E-12 | 8.97E-12 |
| Day 8 | 1.69E-13 | 1.19E-15 | 4.05E-15 | 4.76E-16 | 5.75E-12 | 6.10E-12 |
| Day 9 | 1.29E-13 | 1.43E-15 | 4.52E-15 | 4.76E-16 | 3.21E-12 | 3.49E-12 |
| Day 10 | 1.40E-13 | 1.90E-15 | 4.52E-15 | 7.14E-16 | 4.62E-12 | 4.96E-12 |
| Day 11 | 9.50E-14 | 1.67E-15 | 5.00E-15 | 9.52E-16 | 2.00E-12 | 2.15E-12 |
| Day 12 | 6.24E-14 | 1.67E-15 | 3.81E-15 | 4.76E-16 | 2.30E-12 | 2.50E-12 |
| Day 13 | 5.95E-14 | 2.38E-15 | 4.76E-15 | 4.76E-16 | 1.49E-12 | 1.63E-12 |
| Day 14 | 5.98E-14 | 1.67E-15 | 5.24E-15 | 4.76E-16 | 2.42E-12 | 2.60E-12 |
| Day 15 | 7.14E-14 | 1.43E-15 | 5.48E-15 | 4.76E-16 | 2.64E-12 | 2.84E-12 |
| Day 16 | 6.40E-14 | 1.90E-15 | 3.57E-15 | 7.14E-16 | 3.99E-12 | 4.35E-12 |
| Day 17 | 7.26E-14 | 9.52E-16 | 3.81E-15 | 4.76E-16 | 1.66E-12 | 1.90E-12 |
| Day 18 | 9.33E-14 | 1.43E-15 | 2.62E-15 | 4.76E-16 | 6.10E-12 | 6.68E-12 |
| Day 19 | 5.48E-14 | 1.43E-15 | 3.57E-15 | 4.76E-16 | 1.31E-12 | 1.50E-12 |
| Day 20 | 2.31E-15 | 1.26E-16 | 1.72E-16 | 2.29E-17 | 6.64E-14 | 7.75E-14 |

Table 6: Risk of Inhalation Air Pollutants in Bonny Island during Wet Season

| Period, day | Parameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | CO | NOX | O3 | SO2 | PM2.5 | PM10 |
| Day 1 | 1.09E-13 | 2.62E-15 | 4.76E-15 | 7.14E-16 | 1.04E-12 | 1.15E-12 |
| Day 2 | 1.21E-13 | 3.10E-15 | 4.76E-15 | 9.52E-16 | 1.30E-12 | 1.38E-12 |
| Day 3 | 1.41E-13 | 1.43E-15 | 4.52E-15 | 7.14E-16 | 4.42E-12 | 4.70E-12 |
| Day 4 | 1.21E-13 | 1.19E-15 | 6.43E-15 | 7.14E-16 | 7.33E-13 | 7.95E-13 |
| Day 5 | 1.53E-13 | 1.43E-15 | 2.86E-15 | 4.76E-16 | 4.14E-12 | 4.36E-12 |
| Day 6 | 1.25E-13 | 9.52E-16 | 4.29E-15 | 4.76E-16 | 2.67E-12 | 2.83E-12 |
| Day 7 | 1.36E-13 | 1.19E-15 | 4.05E-15 | 7.14E-16 | 1.52E-12 | 1.62E-12 |
| Day 8 | 1.82E-13 | 7.14E-16 | 3.81E-15 | 4.76E-16 | 2.88E-12 | 3.04E-12 |
| Day 9 | 1.29E-13 | 1.19E-15 | 6.43E-15 | 4.76E-16 | 1.19E-12 | 1.26E-12 |
| Day 10 | 1.42E-13 | 1.43E-15 | 4.53E-15 | 9.52E-16 | 2.80E-12 | 2.95E-12 |
| Day 11 | 1.95E-13 | 7.14E-16 | 2.62E-15 | 7.14E-16 | 9.12E-12 | 9.60E-12 |
| Day 12 | 1.42E-13 | 1.90E-15 | 3.81E-15 | 4.76E-16 | 1.94E-12 | 2.00E-12 |
| Day 13 | 1.83E-13 | 1.19E-15 | 2.86E-15 | 7.14E-16 | 3.94E-12 | 4.13E-12 |
| Day 14 | 1.55E-13 | 4.76E-16 | 2.86E-15 | 7.14E-16 | 4.91E-12 | 5.20E-12 |
| Day 15 | 1.35E-13 | 1.67E-15 | 4.05E-15 | 4.76E-16 | 1.40E-12 | 1.47E-12 |
| Day 16 | 1.25E-13 | 7.14E-16 | 5.71E-15 | 4.76E-16 | 1.39E-12 | 1.46E-12 |
| Day 17 | 1.24E-13 | 4.76E-16 | 4.05E-15 | 4.76E-16 | 2.29E-12 | 2.40E-12 |
| Day 18 | 1.26E-13 | 7.14E-16 | 4.76E-15 | 4.76E-16 | 2.19E-12 | 2.32E-12 |
| Day 19 | 1.80E-13 | 9.52E-16 | 4.29E-15 | 9.52E-16 | 2.82E-12 | 2.93E-12 |
| Day 20 | 6.88E-15 | 5.72E-17 | 3.09E-16 | 2.29E-17 | 1.28E-13 | 1.32E-13 |

4.  Conclusion and Recommendation

4.1 Conclusion: The exposure quantification of CO, NOX, O3, SO2, PM2.5, and PM10for inhalation air indicated that CO, NOX, O3, SO2, PM2.5, and PM10in the air sample were ranging from 9.50E-08 to 1.09E-07, 9.52E-10 to 1.19E-09, 5.48E-09 to 2.62E-09, 9.52E-10 to 4.76E-10, 6.40E-06 to 1.03E-05, and 8.97E-07 to 1.12E-05 µg/m3  during dry and wet seasons. These results were far below the annual threshold of 0.004, 10, 60, 40, 5, and 15 µg/m3 as recommended by the World Health Organisation. In addition, the calculated risk of inhalation of CO, NOX, O3, SO2, PM2.5, and PM10ranging from 6.88E-15 to 1.18E-13, 5.72E-17 to 1.19E-15, 3.09E-16 to 2.14E-15, 2.97E-17 to 4.76E-16, 6.64E-14 to 1.03E-11, and 7.75E-14 to 1.12E-11, were far above the USEPA acceptable risk of 1X10-6 in the study location for the 20 days during dry and wet seasons.

4.2 Recommendation: Further research should be carried out on the effects of air dispersion on particulate matter and gaseous pollutants.

Disclaimer (Artificial intelligence)

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

REFERENCES

- Abegunde, A. A., & Omisore, E. O. (2003, November). The role of urban and regional planning in controlling environmental degradation in Nigeria. In Proceedings of the 11th Annual Conference of Environment and Behavioural Association of Nigeria (EBAN), 65-72.

- Adejunon, (2012). Rainfall season ability in the Niger Delta Belt, Nigeria. Journal of geographical Regional Plan; 5 (2), 51-60.

- Amiriheobu, F. I., George, D. F. C., George, O., & Osah, C. (2020). Environmental Pollution and Death Toll in Bonny Island: A Correlative Study of Ahmed Yerima's Hard Ground and the st Nigerian Development in the 21 Century. International Journal of Scientific Research in Educational Studies & Social Development, 4(1), 163 – 173.

- Bourdrel T., Bind M.A., Bejot, Y., M., Morel, O. & Argacha, J.F (2017). Cardiovascular effects of air pollution. Arch Cardio vase

- Burnett, R.T; Pope, A; III, Ezzati, M; Olives, C; Lim, S.S; Melta, S; Shin, H.H; Singh, G; Hubbell, B; Brauer, M. (2014). An Integrate Risk Function for Estimating the Global Burden of Disease Attributable to Ambient Fine Particulate Matter Exposure. Environ Health Perspect, 2014, 122, 4.

- Chang KH, Chang MY, Muo CH et al (2015) Exposure to air pollution increases the risk of osteoporosis: a nationwide longitudinal study. Medicine (Baltimore) 94(17):e733. doi:10.1097/MD.0000000000000733

- Ede; D. Edokpa and A. Oluleye “Aspects of Air Quality Status of Bonny Island, Nigeria attributed to an LNG plant”. Energy  & Environment 22 (Vol. 22, No. 2 / October 2011): 891 – 910. DOI: 10:1260/0958 – 305x: 7.891

- Emmanouil, C; Drositi, E; Vasilatou, V; Diapouli, E; Krikonis, K; Elefteriadis, K; Kungolos, A; (2016) Study on particulate matter air pollution, source origin and human health risk based of PM10 metal content in Volos City, Greece, Toxicol. Environ; Chem. 2016, 99, 261-709 (Cross Ref).

- Guarneri & Balms (2014) Outdoor air pollution and asthma. Lancet. 383,1581-92. Doi: 10,1016/50140-6736 (14) 60617.6

- Guney, M; Zagury, G.J. Dogab, N; & Onay, T.T (2010). Exposure assessment and risk characterization from trace elements following soil ingestion by children exposed to playgrounds parks and picnic areas. Journal of Hazardous Materials, 182 (1-3), 656-664.

- Guo LJ, Zhao A, Chen RJ, Kan HD, Kuang XY (2014) Association between ambient air pollution and outpatient visits for acute bronchitis in a Chinese City. Biomed Environ Sci 27(11):833–840. doi:10.3967/bes2014.121

- Harrison, R.M, Bousiotis, D; Mohorjy, A.M; Alkhalaf, A.K; Shamy, M; & Alghamdi, M. (2017). Health risk associated with airborne particulate matter and its components in Jeddah Saudi Arabia. Science of the Total Environment, 590, 531-539.

- Hoftmann, B. Moebus, S., Molenkamp, S., Stang, A., Lehmanm, N. & Dragano, N. (2007). Residential exposure to traffic is associated with coronary atherosclerosis. Circulation. 116, 489-496. Doi: 10.1161 / CIRCULATION AHA.107.693622

- https://www.nlng.com/documents/2024_NLNG_FACTS_FIGURESB_compressed.pdf

- Huang, M, Wang, W., Chan, C.Y, Cheng K.C, Man, Y.B, Wang, X; Wong, M.H. (2014) Contamination and Risk Assessment (based on bio accessibility via ingestion and inhalation) of metal (liod) in outdoor and indoor particles from urban centers of Guangzhou China Sci, Total Environ, 2014, 479-480, 117-124. (Cross Ref).

- Jackson, M.M. (2005). Roadside concentration of gaseous and particulate matter pollutants and risk assessment in dar-Es-Salaam, Tazania. Environ Monitor Access, 2005, 104, 385-407 (Cross ref).

- Jevtić M, Dragić N, Bijelović S, Popović M (2014) cardiovascular diseases and air pollution in Novi Sad, Serbia. Int J Occup Med Environ Health 27(2):153–164. doi:10.2478/s13382-014-0239-y

- Jiang, x. Q., Mei, X.D. & Feng, D. (2016). Air pollution and chronic airway diseases: what should people know and do? J Thorac D is ., 8, E 31 – 40.

- Katholi, R.E & Couri, D.M (2011). Left ventricular hypertrophy major risk factor in patients with hypertension: update and practical clinical application Int. J. Itinerates, 2011, 495349. Doi: 10.4061/2011/495349

- Kim JT, Lee HK (2014) Metabolic syndrome and the environmental pollutants from mitochondrial perspectives. Rev Endocr Metab Disord 15(4):253–262. doi:10.1007/s11154-014-9297-5

- Kloog  et al (2013) I. Ridgway, B., Koutrakis, P., Coull, B.A & Schwartz, J.D (2013). Long and short-term exposure to PM2.5 and mortality using novel exposure models, Epidemiology. 24, 555 – 61. Doi: 10. 1097/EDE.OBO1 3e 318294be99.

- Kurt O.K, Zhang, J & Pinkerton, K.E (2016). Pulmonary health effects pf air pollution. Curr Opin pulm med. (2016) 22:138-43. Doi: 1.1097/MCP.0000000000000248

- Leavy, R.J (2015). Carbon monoxide pollution and neurodevelopment: a public health concern. Neurotoxical teratol 49:31-10: doi: 1016/j.ntt. 2015.03.001

- Lee FY, Chen WK, Lin CL, Kao CH (2015) Carbon monoxide poisoning and subsequent cardiovascular disease risk: a nationwide population-based cohort study. Medicine (Baltimore) 94(10):e624. doi:10.1097/MD.0000000000000624

- Li, Y; Zhang, Z, Liu, H; Zhou, H; Fan, Z; Lin, M; (2016 b). Characteristics, sources and health risk assessment of toxic heavy metals in PM2.5 at megacity of southwest China. Environmental Geochemistry and Health, 38 (2), 353 – 362.

- Lim, S.S; Vos, T; Flaxman, A.D; Danaei, G; Shibuya, K; Adair-Rahani, H; Almazaroa, M.A, Amann, M; Anderson, H.R; Andrews, K.C. (2010). A comparative risk assessment of burden disease and injury attributable to 67 risk factors and risk factor clusters in 21 regions, 1990 – 2010. A systematic analysis for the global burden of disease study 2010. Lancet 2012, 380, 224 – 2260.

- Manisalidis, I., Stavropoulou, E., Stavropoulos, A., & Bezirtzoglou, E. (2020). Environmental and Health Impacts of Air Pollution: A Review. Frontier. Public Health 8(14). 1 - 13. doi: 10.3389/fpubh.2020.00014

- Manisalidis, I., Stavropoulou, E., Stavropoulos, A., & Bezirtzoglou, E. (2020). Environmental and Health Impacts of Air Pollution: A Review. Frontier. Public Health 8(14). 1 - 13. doi: 10.3389/fpubh.2020.00014

- Nnanyelugo, O. & Godwin, O. N. (2012). Press coverage of environmental pollution in the Niger Delta region of Nigeria. A content analysis of the guardian, Vanguard, Daily Sun and Thisday Newspapers, 1OSR Journal of Humanities and Social Sciences, 3, (2), pp. 34-46

- Nurmatov UB, Tagiyeva N, Semple S, Devereux G, & Sheikh A (2015) Volatile organic compounds and risk of asthma and allergy: a systematic review. Eur Respir Rev 24(135):92–101. doi:10.1183/09059180.00000714

- Pachoulis et al (2022). “Health risks resulting from short  term exposure to an aggravated environment in population groups living is an industrial area”.

- Prakash, J; Lohia, T; Mandariya, A.K; Habib, G; Gupta, T & Gupta S.K. (2018). Chemical characterization and quantitative assessment of source-specific health risk of trace metals in PM10 at a road site of Delhi, India. Environmental Science and Pollution Research, 25(9), 8747 – 8764.

- Sridevi J; Atahar P; and Gurdeep, S; (2019) Trace element characterization of fine particulate matter and assessment of associated health risk in mining area, transportation routes and institutional area of Dhanbad, India. Environ Geochem Health @ Springer Nature B.V 2019.

- Stamatelopoulou, A; Dasopoulou, M; Bairachtari, K; Karavoltsos, S; Sakellari, A; Maggos, T; (2021). Contamination and Potential Risk Assessment of Polycyclic Aromatic Hydrocarbons (PAHs) and Heavy Metals in House settled Dust collected from Residence of young children. Appl. Sci. 2021, 11, 1479 (Cross Ref).

- U.S. EPA. Risk Assessment Guidance for Superfund: Volume 1 – Human Health Evaluation Manual (Part A) (Interim Final), EPA/540/1e89/002; Office of Emergency and Remedial Response, U.S. Environmental Protection Agency: Washington, DC, USA, 1989.

- Ukpere, D. R. T., Clifford, A. B., Ojule, E. S. C. & Ottah, C. R. (2018). Impacts of air pollution in the Niger Delta Region of Nigeria, IIARD International Journal of Geography and Environmental Management, 4 (4), pp. 13-27

- WHO, (2014) Burden of Disease from the Joint Effects of Household and Ambient Air Pollution for 2012, World Health Organization, 2014, 25, March 2014, http:// www.who.int/phe/health_topics/outdoorair/databases/FINAL_HAP_AAP_ BoD_24March2014.pdf?ua¼1.

- WHO, (2019). Health Risks of Ozone From Long-Range Transboundary Air Pollution. Available online at: http://www.euro.who.int/data/assets/pdf_file/0005/78647/E91843.pdf(accessed August 15, 2019)

- WHO, Regional Office of Europe (2000). Available online at: https://euro.whoint/_data/assets/pdf_file/0020/123086/AQG2ndEd_7_4sulfuroxid.pdf

- Esievo, L., Bright, C. A., & Chukwu, E. O. (2024). Assessment of Suspended Particulate Matter and Prevailing Wind Direction in Bonny Industrial Area of Nigeria.

- Watts, M. (2024). Enclaves of Exception: Special Economic Zones and Extractive Practices in Nigeria. International Journal of African Historical Studies, 57(3).

- Finebone, P. P. (2022). Industrialization and its Impact on Family Livelihood in Bonny, Rivers State, Nigeria. Fl/Otuoke Journal of Sociology and Anthropology (SOFFUOJOSA), 2(3&4).

- Akintoye, O. A., Alaga, O. T., Nkpena, C. O., Harrison, U. E., Uka, U. S., Omoogun, R., ... & Oka, E. M. (2024). A Retrospective Assessment of the Nigeria Liquefied Natural Gas Project and Household Members' Employment in Bonny Local Government Area, Rivers State. Journal of Environmental and Tourism Education Vol, 7(2).

- Jumbo, A., & Ihuah, P. W. (2024). Impact Of Oil And Gas Industry Location On Residential Properties Values In Bonny Island, Rivers State. International Journal of Innovative Environmental Studies Research, 12(1), 19-28.

- Minet, L., Azargoshasbi, F., Franklin, M., Schade, G. W., McGregor, M. J., McInnes, K., & Takaro, T. K. (2025). Analysis of Flaring Activity at Liquefied Natural Gas (LNG) Export Facilities Worldwide. Environmental Science & Technology, 59(38), 20357-20366.

- Kposoruchi, A. S., & Dumka, N. J. (2022). Evaluation of the levels and associated health risk of air pollutants in selected industrial zones in Port Harcourt, Nigeria. International Journal of Chemistry Studies.6(2),95-100

- Singh, V., Srivastava, R. K., & Bhatt, A. K. (2025). Effects of Air Pollution on Life. In Battling Air and Water Pollution: Protecting Our Planet's Vital Resources (pp. 43-60). Singapore: Springer Nature Singapore.