Original Research Article

The invisible threat of pollution in drinking water and impact on human health in Chhindwara district MP, India

ABSTRACT

The clean and pure drinking water is the basic need of human civilization. In this research paper tries to determine the pollution level and quality of drinking water of the chhindwara district.  Drinking water in Chhindwara district various blocks were carried out and their biological effect on public health. This work covering  physicochemical parameters alongside temperature, pH, turbidity, total hardness, alkalinity, TDS, COD, BOD, and DO, all measured by various testing methods across different sampling locations, following standard protocols. All data were evaluated against the acceptable levels set by the Bureau of Indian Standards (BIS) and the World Health Organization (WHO) regarding the human health benefits of consuming clean, fresh water. This research was conducted to assess the quality of drinking water and the levels of heavy metals also in Chhindwara district, which will be drinkable or not.

Keywords: Heavy metals, Drinking water quality, physicochemical parameters, World Health Organisation.

INTRODUCTION

Water contamination is debased water, harmful to people or the climate. It happens when unsafe synthetic substances or microorganisms or now and again essentially trash or a lot of residue sullies a stream, waterway, lake, wetland, groundwater spring, sea, so, any waterway. Anthropogenic warm contrasts brought about by the release of high temp water from power plants and industrial facilities straightforwardly into an untamed water source is additionally viewed as water contamination as it diminishes oxygen levels and adjusts the species that can make due there, frequently welcoming non-local species attack, modifying the natural pecking order and regularly lessening species biodiversity. The sacred Ganges River takes the prize for the world’s most polluted river, carrying sewage, trash, animal carcasses and wasted food. The single biggest reason for water pollution in India is urbanization at an uncontrolled rate. The treatment and disposal of wastewater has also been a major issue in this regard. The areas near rivers have seen plenty of towns and cities come up and this has also contributed to the growing intensity of problems. Uncontrolled urbanization in these areas has also led to generation of sewage water. In the urban areas water is used for both industrial and domestic purposes from water bodies such as rivers, lakes, streams, wells, and ponds. Worst still, 80% of the water that we use for our domestic purposes is passed out in the form of wastewater. In most of the cases, this water is not treated properly and as such it leads to tremendous pollution of surface level freshwater.

2. Sources of Water Pollution: The two chief sources of water pollution can be seen as:

2.1 Point Source: Point source refers to the pollutants that belong to a single source. An example of this would be emissions from factories into the water. Point source

2.2 Non-Point Source: Non point refers to the pollutants emitted from multiple sources. Contaminated water after rains that has travelled through several regions may also be considered as a non-point source of water pollution. Non-point source pollution refers to diffuse contaminant that does not originate from a single discrete source. NPS pollution is often accumulative effect of small amounts of contaminants gathered from a large area. The leaching out of nitrogen compounds from agricultural land which has been fertilized is a typical example.

3. Causes of Water Pollution:

3.1 Agriculture and Nutrient Pollution: Around the world, agriculture is the leading cause of water degradation. Every time it rains, fertilizers, pesticides, and animal waste from farms and livestock operations wash nutrients and pathogens such bacteria and viruses into our waterways. Nutrient pollution, caused by excess nitrogen and phosphorus in water or air, is the number one threat to water quality worldwide and can cause algal blooms, a toxic soup of blue-green algae that can be harmful to people and wildlife. Some waste water, fertilizers and sewage contain high levels of nutrients. If they end up in water bodies they encourage algae and weed growth in the water bodies, this will make the water undrinkable, and even clog filters. Too much algae will also use all the oxygen starvation

3.2 Sewage and Wastewater: Used water is wastewater. It comes from our sinks, showers, and toilets (think sewage) and from commercial, industrial, and agricultural activities (think metals, solvents, and toxic sludge). The term also includes storm water runoff, which occurs when rainfall carries road salts, oil, grease, chemicals, and debris from impermeable surfaces into our waterways.

3.3 Surface Water Pollution: Surface water includes natural water found on the earth’s surface like rivers, lakes, lagoons and oceans. Hazardous substances coming into contact with this surface water, dissolving or mixing physically with the water can be called surface water pollution. For example, industries emptying oil spills and chemical waste into water bodies are also key contributors.

3.4 Oxygen Depletion: Water bodies have microorganism. These include aerobic and anaerobic organisms, when too much biodegradable matter (things that easily decay) end up in water, it encourages more microorganisms growth, and they use up more oxygen in the water. If oxygen is depleted, aerobic organisms die, and anaerobic organisms grow more to produce harmful toxins such as ammonia, sulfides.

3.5 Ground Water Pollution: When humans apply pesticides and chemicals to soils, they are washed deep into the ground by rain water. This gets to underground.

Maximum and minimum allowed levels of certain physicochemical parameters and heavy metals in water as per BIS 201211 and WHO 2008. 12

TABLE-1 Permissible limits of various parameters of water

| Serial no. | Parameter | Unit | Minimum permissible limit | Maximum permissible limit | Reference |
| --- | --- | --- | --- | --- | --- |
| 1. | pH | - | 6.5 | 8.5 | 11 |
| 2. | Turbidity | NTU | 1 | 5 | 12 |
| 3. | Color | TCU | 5 | 15 | 12 |
| 4. | TDS (Total Dissolve Solid) | mg/ L | 200 | 600 | 11 |
| 5. | DO (Dissolve Oxygen) | mg/ L | 3.5 | 8.0 | 11 |
| 6. | Hardness | mg/ L | 300 | 600 | 12 |
| 7. | BOD(Biochemical Oxygen Demand) | mg/ L | - | 2 | 11 |
| 8. | COD(Chemical Oxygen Demand) | mg/ L | - | - | 11 |
| 9. | Alkalinity | mg/ L | 200 | 600 | 11 |
| 10. | Iron | mg/ L | 0.3 | 1 | 11 |
| 11. | Nickel | mg/ L | 0.07 | No Relaxation | 11 |
| 12. | Zink | mg/ L | 5 | 15 | 11 |
| 13. | Copper | mg/ L | 0.05 | 1.5 | 11 |
| 14. | Cadmium | mg/ L | 0.01 | No Relaxation | 12 |
| 15. | Lead | mg/ L | 0.05 | No Relaxation | 11 |
| 16. | Chromium | mg/ L | 0.05 | No Relaxation | 11 |
| 17. | Arsenic | mg/L | 0.01 | No Relaxation | 12 |
| 18 | Manganese | mg/ L | 0.1 | 0.3 | 11 |
| 19 | Cobalt | mg/L | 0.001 | No Relaxation | 12 |

EXPERIMENTAL METHODOLOGY

Study area

Chhindwara district is a town situated in the southern region of Madhya Pradesh. It ranges from 21.28 to 22.49 degrees in the North (longitude) and from 78.40 to 79.24 degrees in the East (latitude), covering an area of 11,815 km². This region is bordered by the plains of Nagpur District (in Maharashtra State) to the South, Hoshangabad and Narsinghpur Districts to the North, Pandhurna District to the West, and Seoni District to the East. The research area encompasses 13 tehsils (Chhindwara, Parasia, Junnardev, Amarwada, Chourai, Sousar, Pandhurna, Bichhua, Mohkhed, Tamia, Umreth, Chand, and Harrai) from the Chhindwara district

General Testing Procedure

In this research work various scientific test were done by different procedure and related instrument or analytical method. The heavy metal concentration detect by AAS (Atomic Absorption Spectrophotometer), TDS by Turbiditimeter, PH by PH meter, Alkalinity and Acidity by conductivity meter. DO by Analytical procedure, BOD and COD by BOD incubator, Hardness by Laboratory analytical method and other test by testing methods.

Pic  1.  Location of the study areas in Chhindwara district

Sample collection

13 samples were collected from all the blocks in the summer, rainy, and winter seasonsof 2024.The bottles were cleaned and rinsed with sample water before sampling.These samples were taken fromplaces where more people use water for drinking. Physicochemical parameters like Temperature, pH, turbidity, Total hardness, Alkalinity, TDS, Heavy Metals(Co, Ni, Fe, Mn, Cu, Zn, Cr, Cd, PbandAs), COD, BOD and DO. Various parameters were tested with the assistance of our laboratory, the Madhya Pradesh Council of Science and Technology (MPCST), Bhopal, the Maulana Azad National Institute of Technology (MANIT), Bhopal, and the PHE Department,Chhindwara.

RESULT AND DISSCUSSION

The results ​​obtained after testing the physicochemical parameters, including pH, turbidity, total hardness, alkalinity, DO, COD, BOD, TDS and heavy metals (Co, Ni, Fe, Mn, Cu, Zn, Cr, Cd, Pb, and As) of the drinking water samples are given in Table -2, 3.

Discussion of Heavy Metals

Iron

The shortage of iron causes a disease called "anaemia", and prolonged consumption of drinking water with high concentrations of iron may lead to a liver disease called hemosiderosis.13,14According to the First sampling table, the lowest iron value among the samples analysed was 0.693 mg/L from the Amarwada blocks. The highest iron value of 3.018 mg/L was recorded in the Tamia block's drinking water sample; the rest of the block has a value of zero. According to the second sampling table, the lowest iron value among the samples analysed was 0.481 mg/L from the Tamia blocks. The highest iron value of 3.917 mg/L was recorded in the drinking water sample from the Chand block. The iron value of the Umreth block is 3.237 mg/L, and the rest of the block has an Iron value of zero. According to the third sampling table, the lowest iron value among the samples analysed was 0.001 mg/L from Tamia, Chourai, Sousar, Amarwada and Chand blocks. The highest iron value of 0.003mg/L was recorded in the drinking water sample from the Pandhurna block. The iron value of the Harrai and Junnardeo block is 0.002 mg/L, and the rest of the block has an Iron value of zero. However, according to the BIS, the minimum value of iron in drinking water should be between 0.3mg/L maximum value of iron should be 1 mg/L.

First sampling data collection, water samples were taken from all the blocks in May 2024. Second sampling data collection: Water samples were taken from all the blocks in August 2024. Third sampling data collection: Water samples were taken from all the blocks in December 2024. The results obtained after testing those samples are given in Table 2, 3

Table –2,  Heavy metals testing result of the water samples.

| S.NO | Heavy metals | Sampling Time | Amarwada | Bichhua | Chand | Chhindwara | Chourai | Harrai | Junnardeo | Mohkhed | Pandhurna | Parasia | Sousar | Tamia | Umreth |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Iron (mg/L) | 1st | 0.639 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 3.018 | 0.0 |
|  |  | 2nd | 0.0 | 0.0 | 3.917 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.481 | 3.237 |
|  |  | 3rd | 0.001 | 0.0 | 0.001 | 0.0 | 0.001 | 0.002 | 0.002 | 0.0 | 0.003 | 0.0 | 0.001 | 0.001 | 0.0 |
| 2 | Copper (mg/L) | 1st | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.254 |
|  |  | 2nd | 0.003 | 0.0 | 0.027 | 0.0 | 0.0 | 0.0 | 0.015 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.003 |
|  |  | 3rd | 0.0 | 0.001 | 0.0 | 0.0 | 0.001 | 0.002 | 0.0 | 0.0 | 0.003 | 0.002 | 0.0 | 0.0 | 0.002 |
| 3 | Cadmium (mg/L) | 1st | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.005 | 0.0 |
|  |  | 2nd | 0.005 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.005 | 0.005 | 0.0 | 0.048 | 0.0013 | 0.0 |
|  |  | 3rd | 0.002 | 0.0 | 0.002 | 0.002 | 0.003 | 0.003 | 0.0 | 0.002 | 0.003 | 0.003 | 0.001 | 0.001 | 0.002 |
| 4 | Lead (mg/L) | 1st | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  |  | 2nd | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  |  | 3rd | 0.0 | 0.001 | 0.0 | 0.0 | 0.003 | 0.0 | 0.0 | 0.0 | 0.004 | 0.002 | 0.004 | 0.0 | 0.001 |
| 5 | Heavy Metals ( Cd, Zn, Mn, Ni, Co, As) | 1st | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  |  | 2nd | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  |  | 3rd | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |

Copper

Copper generates oxygen radicals via a Fenton-type reaction,15 and many investigators have hypothesised that excess copper might cause cellular injury via an oxidative pathway, giving rise to enhanced lipid peroxidation,thiol oxidation and, ultimately, DNA damage 16,17,18The highest copper value of 0.254 mg/L was recorded in the drinking water sample from the Umreth block. The lowest copper value among the samples analysed was 0 mg/L from all blocks. According to the secondsampling table, the lowest copper value among the samples analysed was 0.003 mg/L from the Amarwada blocks. The highest copper value of 0.027 mg/L was recorded in the drinking water sample from the Chand block. The copper value of the Umreth and Junnardeo block is 0.015 mg/L, and the rest of the block has a copper value of zero. According to the third sampling table, the lowest copper value among the samples analysed was 0.001 mg/L from the Bichhua and Chouraiblocks. The highest copper value of 0.003 mg/L was recorded in the drinking water sample from the Pandhurnablock. The copper value of the Harrai, Parasia and Umreth block is 0.002 mg/L, and the rest of the block has a copper value of zero. However, according to the BIS, the minimum value of copper in drinking water should be between 0.05mg/L maximum value of copper should be 1.5 mg/L.

Cadmium

In humans, Cd exposure can result in a variety of adverse effects, such as renal and hepatic dysfunction, pulmonary oedema, and testicular damage.19The lowest cadmium value among the samples analysed was 0 mg/L from all blocks. The highest value ofcadmium, 0.005 mg/L, was recorded in the drinking water sample from the Tamia block. According to the second sampling table, the lowest cadmium value among the samples analysed was 0.005 mg/L from Amarwada, Mohkhed and Pandhurna blocks.

The highest cadmium value of 0.048 mg/L was recorded in the drinking water sample from the Sousarblock. The cadmium value of the Tamia block is 0.013 mg/L, and the rest of the block has a cadmium value of zero. According to the third sampling table, the lowest cadmium value among the samples analysed was 0.001 mg/L from the Sousar and Tamia blocks. The highest cadmium value of 0.003 mg/L was recorded in the drinking water sample from the Parasia, Chourai and Harrai blocks. Cadmium value of Chhindwara, Mohkhed, Amarwada, Chand and Umreth block is 0.002 mg/L, and the rest of the block has a cadmium value of zero. However, according to the WHO, the minimum value of cadmium in drinking water should be 0.01mg/L.

Lead

In the First and second tables, the value of lead is zero, but according to the third sampling table, the lowest lead value among the samples analysed was 0.001 mg/L from the Bichhua and Umrethblocks. The highest lead value of 0.004 mg/L was recorded in the drinking water sample from the Pandhurna and Sousar blocks. The lead value of Chourai block is 0.003 mg/L, the lead value of Parasia block is 0.002 mg/L, and the rest of the block has a lead value of zero.However, according to the BIS, the minimum value of lead in drinking water should be 0.05mg/L.

Heavy metal ions can cause serious health problems such as liver and kidney damage, skin disorders, cognitive impairment, and even cancer. To prevent the harmful effects of these toxic metals, it is important to find an eco-friendly and cost-effective method to remove heavy metal ion contamination from wastewater.20Testing of drinking water through AAS in all the blocks revealed that the value of heavy metals like chromium, zinc, manganese, nickel, cobalt and arsenic was 0 mg/L.

Discussion of Physicochemical Parameter

pH

The lowest pH value of drinking water in the study area was 7.57, and the highest pH value was 8.30. According to the second sampling table, the lowest pH value among the samples analysed was 6.77 from Umreth block. The highest pH value of 7.55 was recorded in the drinking water sample from the Pandhurna block. According to the third sampling table, the lowest pH value among the samples analysed was 7.10 from Umreth block. The highest pH value of 8.20 was recorded in the drinking water sample from the Chouraiblock. According to BIS, the pH of drinking water should be between 6.5-8.5,whereas everywhere was slightly alkaline 21

Total dissolved solids

The palatability of water with a TDS level of less than 500 mg/L is generally considered to be good. When the TDS levels are above 1000 mg/L, drinking water becomes largely unpalatable.TDS greater than 1200 mg/L may be objectionable to consumers and could have impacts on those who need to limit their daily salt intake, e.g. severely hypertensive, diabetic, and renal dialysis patients.22 The lowest TDS value among the samples analysed was 19.3 mg/L from Tamia block.  The highest TDS value of 765 mg/L was recorded in the drinking water sample from the Parasia block. According to the second sampling table, the lowest TDS value among the samples analysed was 22 mg/L from the Tamia block. The highest TDS value of 462 mg/L was recorded in the drinking water sample from the Sousarblock. According to the third sampling table, the lowest TDS value among the samples analysed was 24 mg/L from the Tamia block. The highest TDS value of 780 mg/L was recorded in the drinking water sample from the Sousarblock. However, according to the BIS, the lowest value of TDS of drinking water should be between 200mg/L and the highest value should be between 600 mg/L.

Turbidity

The dirt in water stimulates the growth of bacteria risks consumer health.23 Many national organisations like the BIS have also set their drinking water standards (Table 1).The lowest turbidity value among the samples analysed was 1 NTU from Bichhua, Mohkhed, Parasia, Amarwada and Umreth blocks.  The highest turbidity value of 9 NTU was recorded in the drinking water sample from the Pandhurna block.

According to the second sampling table, the lowest turbidity value among the samples analysed was 1NTU from Umreth, Bichhua, Parasia, Chand and Sousar blocks. The highest turbidity value of 30 NTU was recorded in the drinking water sample from the Amarwadablock. According to the third sampling table, the lowest turbidity value among the samples analysed was 1 NTU from Umreth, Bichhua, Amarwada, Harrai, Sousar, and Chand blocks. The highest turbidity value of 8 NTU was recorded in the drinking water sample from the Chhindwarablock. However, according to the WHO, the lowest value of turbidity of drinking water should be between 1 NTU and the highest value should be between 5 NTU.

Alkalinity

Alkalinity is one of the important properties of water, which is a measure of the ability of water to neutralise acids.24Alkalinity poses no known health risks to humans.The lowest alkalinity value among the samples analysed was 30 mg/L from the Tamia block.  The highest alkalinity value of 275 mg/L was recorded in the drinking water sample from the Bichhuablock. According to the second sampling table, the lowest alkalinity value among the samples analysed was 35 mg/L from the Tamia blocks. The highest alkalinity value of 485 mg/L was recorded in the drinking water sample from the Sousarblock. According to the third sampling table, the lowest alkalinity value among the samples analysed was 20 mg/L from the Tamia block. The highest alkalinity value of 375 mg/L was recorded in the drinking water sample from the Sousarblock. However, according to the BIS, the lowest value of alkalinity of drinking water should be between 200mg/L and the highest value should be between 600 mg/L.

Hardness

Hard water can reduce the rate of heart disease. There are some negative effects of using hard water, these include wastage of soap and scaling of pipes and boilers.25,26According to the WHO maximum value of total hardness should be 500 mg/L. The lowest hardness value among the samples analysed was 5 mg/L from Tamia block. The highest hardness value of 220 mg/L was recorded in the drinking water sample from the Parasia block. According to the second sampling table, the lowest hardness value among the samples analysed was 65 mg/L from Junnardeo blocks. The highest hardness value of 165 mg/L was recorded in the drinking water sample from the Sousarblock. According to the third sampling table, the lowest hardness value among the samples analysed was 25 mg/L from Tamia blocks. The highest hardness value of 325 mg/L was recorded in the drinking water sample from the Parasiablock. However, according to the BIS, the lowest value of hardness of drinking water should be between 300 mg/L and the highest value should be between 600 mg/L.

Dissolved Oxygen

Dissolved oxygen is said tobefree oxygen in water, which is involved in various biochemical and physiological activities.27,28 The amount of oxygen dissolved in water is an important indicator of water quality and an important factor in water purification.The lowest DO value among the samples analysed was 7.5mg/L from Umreth block.  The highest DO value of 18.2 mg/L was recorded in the drinking water sample from the Tamia block. According to the second sampling table, the lowest DO value among the samples analysed was 11.2 mg/L from Bichhua block. The highest DO value of 14.2 mg/L was recorded in the drinking water sample from the Tamia block. According to the third sampling table, the lowest DO value among the samples analysed was 6.0 mg/L from Bichhua block. The highest DO value of 9.4 was recorded in the drinking water sample from the Chhindwarablock. However, according to the BIS, the lowest value of DO in drinking water should be between 3.5 mg/L and the highest value should be between 8 mg/L.

Biochemical oxygen demand

BOD represents an approximate assessment of biodegradable organic matter content existing in an aquatic ecosystem.29The lowest BOD value among the samples analysed was 0.5 mg/L from the Tamia and Amarwada block.  The highest BOD value of 5.7 mg/L was recorded in the drinking water sample from Chand block. According to the second sampling table, the lowest BOD value among the samples analysed was 0.1 mg/L from Mohkhed blocks. The highest BOD value of 1.6 mg/L was recorded in the drinking water sample from the Pandhurnablock. According to the third sampling table, the lowest BOD value among the samples analysed was 0.3 mg/L from the Chhindwara block. The highest BOD value of 1.3 was recorded in the drinking water sample from the Junnardeoblock. However, according to the BIS, the lowest value of the BOD of drinking water should be between 1 mg/L and the highest value should be between 2 mg/L.

Table 3, Testing results of the various parameters of the water samples.

| S.NO | Physicochemical Parameter | Sampling Time | Amarwada | Bichhua | Chand | Chhindwara | Chourai | Harrai | Junnardeo | Mohkhed | Pandhurna | Parasia | Sousar | Tamia | Umreth |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Turbidity(NTU) | 1st | 1.0 | 1.0 | 2.0 | 7.0 | 8.0 | 3.0 | 5.0 | 1.0 | 9.0 | 1.0 | 7.0 | 0.0 | 1.0 |
|  |  | 2nd | 30.0 | 1.0 | 1.0 | 17.0 | 7.0 | 2.0 | 9.0 | 2.0 | 5.0 | 1.0 | 1.0 | 13.0 | 1.0 |
|  |  | 3rd | 1.0 | 1.0 | 1.0 | 8.0 | 4.0 | 1.0 | 3.0 | 2.0 | 2.0 | 3.0 | 1.0 | 2.0 | 1.0 |
| 2 | pH | 1st | 8.08 | 7.65 | 7.72 | 7.98 | 8.19 | 8.2 | 8.1 | 7.95 | 8.3 | 7.79 | 7.71 | 7.57 | 7.71 |
|  |  | 2nd | 7.32 | 7.3 | 6.7 | 7.5 | 7.05 | 7.9 | 7.1 | 7.4 | 7.55 | 7.16 | 6.8 | 7.53 | 6.77 |
|  |  | 3rd | 7.82 | 7.3 | 7.4 | 7.8 | 8.2 | 8.01 | 7.9 | 7.8 | 7.8 | 7.9 | 7.41 | 7.5 | 7.1 |
| 3 | TDS (mg/L) | 1st | 184.0 | 304.0 | 500.0 | 185.0 | 238.0 | 207.0 | 174.0 | 180.0 | 253.0 | 765.0 | 510.0 | 19.3 | 448.0 |
|  |  | 2nd | 125.0 | 254.0 | 419.0 | 144.0 | 312.0 | 124.0 | 75.0 | 137.0 | 133.0 | 218.0 | 462.0 | 22.0 | 337.0 |
|  |  | 3rd | 242.0 | 416.0 | 769.0 | 294.0 | 286.0 | 273.0 | 184.0 | 227.0 | 251.0 | 988.0 | 780.0 | 24.0 | 647.0 |
| 4 | Alkalinity (mg/L) | 1st | 195.0 | 275.0 | 230.0 | 120.0 | 200.0 | 215.0 | 115.0 | 105.0 | 195.0 | 265.0 | 200.0 | 30.0 | 260.0 |
|  |  | 2nd | 135.0 | 350.0 | 315.0 | 155.0 | 360.0 | 125.0 | 105.0 | 140.0 | 190.0 | 340.0 | 485.0 | 35.0 | 275.0 |
|  |  | 3rd | 120.0 | 265.0 | 320.0 | 170.0 | 200.0 | 160.0 | 150.0 | 125.0 | 190.0 | 240.0 | 375.0 | 20.0 | 280.0 |
| 5 | Total Hardness (mg/L) | 1st | 70.0 | 85.0 | 95.0 | 65.0 | 85.0 | 75.0 | 85.0 | 70.0 | 40.0 | 220.0 | 140.0 | 5.0 | 110.0 |
|  |  | 2nd | 145.0 | 145.0 | 120.0 | 95.0 | 130.0 | 130.0 | 65.0 | 125.0 | 100.0 | 140.0 | 165.0 | 70.0 | 150.0 |
|  |  | 3rd | 100.0 | 135.0 | 170.0 | 105.0 | 95.0 | 110.0 | 105.0 | 120.0 | 115.0 | 325.0 | 165.0 | 25.0 | 175.0 |
| 6 | COD (mg/L) | 1st | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  |  | 2nd | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  |  | 3rd | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 7 | DO(mg/L) | 1st | 14.2 | 17.4 | 15.0 | 12.2 | 18.0 | 21.2 | 14.6 | 18.0 | 13.1 | 12.4 | 16.9 | 18.2 | 7.5 |
|  |  | 2nd | 14.1 | 11.2 | 12.0 | 13.2 | 14.4 | 14.0 | 15.3 | 13.4 | 12.7 | 13.8 | 13.0 | 14.5 | 13.9 |
|  |  | 3rd | 8.0 | 6.0 | 6.7 | 9.4 | 7.9 | 7.2 | 9.0 | 8.7 | 8.0 | 8.9 | 8.8 | 7.3 | 7.5 |
| 8 | BOD (mg/L) | 1st | 0.5 | 3.8 | 5.7 | 3.8 | 1.8 | 2.7 | 4.1 | 1.7 | 2.8 | 2.4 | 1.1 | 0.5 | 2.1 |
|  |  | 2nd | 2.2 | 0.5 | 0.4 | 0.4 | 1.4 | 0.0 | 0.6 | 0.1 | 1.6 | 1.4 | 1.2 | 0.5 | 1.5 |
|  |  | 3rd | 0.7 | 1.0 | 0.5 | 0.3 | 0.9 | 1.2 | 1.3 | 0.4 | 1.1 | 0.6 | 1.0 | 1.2 | 0.8 |
| 9 | Iron (mg/L) | 1st | 0.639 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 3.018 | 0.0 |
|  |  | 2nd | 0.0 | 0.0 | 3.917 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.481 | 3.237 |
|  |  | 3rd | 0.001 | 0.0 | 0.001 | 0.0 | 0.001 | 0.002 | 0.002 | 0.0 | 0.003 | 0.0 | 0.001 | 0.001 | 0.0 |
| 10 | Copper (mg/L) | 1st | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.254 |
|  |  | 2nd | 0.003 | 0.0 | 0.027 | 0.0 | 0.0 | 0.0 | 0.015 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.003 |
|  |  | 3rd | 0.0 | 0.001 | 0.0 | 0.0 | 0.001 | 0.002 | 0.0 | 0.0 | 0.003 | 0.002 | 0.0 | 0.0 | 0.002 |
| 11 | Cadmium (mg/L) | 1st | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.005 | 0.0 |
|  |  | 2nd | 0.005 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.005 | 0.005 | 0.0 | 0.048 | 0.0013 | 0.0 |
|  |  | 3rd | 0.002 | 0.0 | 0.002 | 0.002 | 0.003 | 0.003 | 0.0 | 0.002 | 0.003 | 0.003 | 0.001 | 0.001 | 0.002 |
| 12 | Lead (mg/L) | 1st | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  |  | 2nd | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  |  | 3rd | 0.0 | 0.001 | 0.0 | 0.0 | 0.003 | 0.0 | 0.0 | 0.0 | 0.004 | 0.002 | 0.004 | 0.0 | 0.001 |
| 13 | Heavy Metals ( Cd, Zn, Mn, Ni, Co, As) | 1st | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  |  | 2nd | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  |  | 3rd | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |

Chemical oxygen demand

The most relevant pollution effect of wastewater is organic matter. The pollution level of organic matter in water bodies is roughly estimated by analysing the COD.30 COD is a superior representative of organic matter than BOD because BOD does not provide any information regarding the oxidation state of an organic substance.31 By contrast; COD can overcome the drawbacks obtained by BOD. Thus, COD is considered an important water quality parameter for representing the degree of organic pollution.32 The value of Chemical Oxygen Demand is zero in all blocks, which is correct according to BIS and WHO.

BLOCK-WISE HEAVY METAL EFFECTS ON HUMAN HEALTH

1. Amarwada

- Detected Metals: Fe (0.639 mg/L), Cu (0.003 mg/L), Cd (0.002 mg/L).

- Health Impact: The Fe concentration above permissible limits may cause gastrointestinal irritation, liver dysfunction, and staining of teeth. Presence of Cd can gradually damage the kidneys, weaken bones, and affect calcium metabolism. Trace Cu may cause mild stomach irritation on long exposure. Many of these traits have been observed in the people of Amarwada

2. Bichhua

- Detected Metals: Cu (0.001 mg/L), Pb (0.001 mg/L).

- Health Impact: Although concentrations are low, even minimal Pb exposure can accumulate in the human body, causing neurodevelopmental issues in children, anemia, and hypertension in adults. Continuous exposure to Cu may disrupt liver enzyme functions.Many of these traits have been observed in the people of Bichhua.

3. Chand

- Detected Metals: Fe (3.917 mg/L), Cu (0.027 mg/L), Cd (0.002 mg/L).

- Health Impact: Excess Fe far above WHO guideline may lead to hemochromatosis, joint pain, and liver cirrhosis. Cd presence may increase the risk of kidney damage, bone fragility, and reproductive issues. Cu in excess may trigger nausea, vomiting, and liver toxicity.Many of these traits have been observed in the people of Chand

4. Chhindwara

- Detected Metals: Cd (0.002 mg/L).

- Health Impact: Cd, even in small amounts, is highly toxic and carcinogenic. Its value is in limit according to BIS.

5. Chourai

- Detected Metals: Cd (0.003 mg/L), Pb (0.003 mg/L).

- Health Impact: Both metals exceed WHO limits. Cd exposure may cause nephrotoxicity and chronic bone disease, while Pb may result in neurological impairments, developmental delays in children, and cardiovascular issues in adults.

6. Harrai

- Detected Metals: Cd (0.003 mg/L).

- Health Impact: Long-term Cd intake may lead to brittle bones, kidney damage, and increased risk of cancer. Its bioaccumulation in the body can also disturb hormonal balance.

7. Junnardeo

- Detected Metals: Fe (0.002 mg/L), Cu (0.015 mg/L), Cd (0.005 mg/L).

- Health Impact: Cd concentration is significantly higher than the permissible limit, indicating strong risk of chronic kidney disease. Cu may cause gastric irritation, liver stress, and vomiting if exposure continues.Many of these traits have been observed in the people of Junnardeo.

8. Mohkhed

- Detected Metals: Fe (0.003 mg/L), Pb (0.004 mg/L).

- Health Impact: Pb contamination at this level may cause nervous system damage, reduced IQ in children, learning disabilities, and long-term cardiovascular effects. Fe levels are within limits but may contribute to minor digestive issues if accumulated. Many of these traits have been observed in the people of Mohkhed.

9. Pandhurna

- Detected Metals: Cd (0.048 mg/L – very high), Pb (0.004 mg/L).

- Health Impact: Cd concentration is alarmingly high, which may cause Itai-Itai disease (bone softening, severe pain), kidney failure, and cancer. Pb further adds risks of hypertension, brain damage, and impaired fetal development. This block shows the highest contamination risk.Many of these traits have been observed in the people of Pandhurna

10. Parasia

- Detected Metals: Fe (0.001 mg/L), Pb (0.002 mg/L).

- Health Impact: Pb even in low levels can cause anemia, memory loss, and developmental disorders. Long-term exposure may also trigger reproductive health issues.

11. Sousar

- Detected Metals: Negligible traces of Fe, Cu, Cd.

- Health Impact: The contamination levels are within safe limits, indicating minimal immediate risk, though continuous monitoring is required to avoid bioaccumulation.

12. Tamia

- Detected Metals: Fe (3.018 mg/L), Cu (0.002 mg/L).

- Health Impact:Very high Fe may cause liver and pancreatic disorders, as well as skin pigmentation. Cu traces, if accumulated, may increase the risk of gastrointestinal disturbances and kidney stress.Many of these traits have been observed in the people of Tamia

13. Umreth

- Detected Metals: Fe (3.237 mg/L), Cu (0.002 mg/L), Pb (0.001 mg/L).

- Health Impact: Excess Fe may lead to metabolic disorders, diabetes, and liver dysfunction. Pb, even at low levels, Many of these traits have been observed in the people of Umreth

CONCLUSION

Above data from the first, second, and third sampling, it was concluded that, the various parameters is in limit but some parameters cross the limit of BIS and WHO.

pH - The highest pH value was 8.30 in pandhurna block , whereas the lowest pH value among the samples analysed was 6.77 from Umreth block. Higher value of iron is cause of hemesiderosis, and other disfunction of the body.

Alkalinity - The highest alkalinity value of 485 mg/L was recorded in the drinking water sample from the Sousar block and lowest value is in the Tamia block.

Colour – Drinking water color was the clean and transparent of the all samples.

Temperature – It is normal value of the all samples.

Turbidity - The highest turbidity value of 9 NTU was recorded in the drinking water sample from the Pandhurna block. The lowest turbidity value among the samples analysed was 1 NTU from Bichhua, Mohkhed, Parasia, Amarwada and Umreth blocks.

TDS (Total Dissolve Solid) - The lowest TDS value among the samples analysed was 19.3 mg/L from Tamia block.  The highest TDS value of 765 mg/L was recorded in the drinking water sample from the Parasia block.

Hardness - The lowest hardness value among the samples analysed was 5 mg/L from Tamia block. The highest hardness value of 220 mg/L was recorded in the drinking water sample from the Parasia block.

DO (Dissolve Oxygen) - The highest DO value of 18.2 mg/L was recorded in the drinking water sample from the Tamia block. Lowest DO value among the samples analysed was 6.0 mg/L from Bichhua block.

BOD (Biochemical Oxygen Demand) - The lowest BOD value among the samples analysed was 0.1 mg/L from Mohkhed blocks. The highest BOD value of 5.7 mg/L was recorded in the drinking water sample from Chand block.

COD (Chemical Oxygen Demand) – Nil

Iron - The highest iron value of 3.018 mg/L was recorded in the Tamia block's drinking water sample

Copper - The highest copper value of 0.254 mg/L was recorded in the drinking water sample from the Umreth block.

Cadmium- The highest cadmium value of 0.048 mg/L was recorded in the drinking water sample from the Sousar block. The cadmium value of the Tamia block is 0.013 mg/L,

Lead - The highest lead value of 0.004 mg/L was recorded in the drinking water sample from the Pandhurna and Sousar blocks.

Chromium - Nil

Arsenik - Nil

Mangnise - Nil

Nickel - Nil

Cobalt - Nil

Zink - Nil

# COMPETING INTERESTS DISCLAIMER:

Authors have declared that they have no known competing financial interests OR non-financial interests OR personal relationships that could have appeared to influence the work reported in this paper.

Disclaimer (Artificial intelligence)

1. In this research paper I am not used NO generative AI technologies such as Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

2. I am not used any type AI tools for editing or generating language of the research paper.

REFERENCES

- World Health Organization. (2017). Guidelines for Drinking-Water Quality: Fourth Edition Incorporating the First Addendum. World Health Organization. https://iris.who.int/handle/10665/254637

- World Health Organization & United Nations Children's Fund. (2017). Progress on drinking water, sanitation and hygiene: 2017 update and SDG baselines. World Health Organization. https://www.who.int/publications/i/item/9789241512893

- World Health Organization & United Nations Children’s Fund. (2022). Progress on sanitation and drinking-water: 2022 update. https://www.jmp.who.int/sites/default/files/2010/JMP_2022_report_en.pdf

- L. Gao, J. Chen, C. Tang, et al. Environ Sci Process Impacts17:1769-1782, (2015). https://doi.org/10.1039/C5EM00156K

- L. Tabandeh, A. Dargahi, M. Mohammadi, A. Azizi, J Occup Environ Health 2(1): 72-81, (2016).

- Nejatijahromi, Z., Nassery, H. R., Nakhaei, M., &Alijani, F. (2018). Assessment of the quality of groundwater for drinking purposes in Varamin aquifer: heavy metals contamination. *Iranian Journal of Health and Environment*, *10*(4), 559-572. https://www.cabidigitallibrary.org/doi/10.1079/cabicompendium.20193007001

- ParsiMehr, M., Hekmati, M., & Shayesteh, K. (2020). Investigation of heavy metals in drinking water: A systematic review in Iran. Journal of Advances in Environmental Health Research. https://doi.org/10.22102/jaehr.2020.232374.1170

- United Nations. (2018). The 2030 Agenda and the Sustainable Development Goals: An opportunity for Latin America and the Caribbean. Economic Commission for Latin America and the Caribbean (ECLAC). https://www.cepal.org/sites/default/files/events/files/2030_agenda_and_the_sdgs_an_opportunity_for_latin_america_and_the_caribbean.pdf

- Li, Y., & Trush, M. A. (1993). DNA damage resulting from the oxidation of hydroquinone by copper: role for a Cu(II)/Cu(I) redox cycle and reactive oxygen generation. Carcinogenesis, 14(7), 1303-1311. https://doi.org/10.1093/carcin/14.7.1303

- Tinkov, A. A., Filippini, T., Ajsuvakova, O. P., Skalny, A. V., Skalnaya, M. G., Solovyev, N. D., Gatiatulina, M. A., Popova, M. A., Aschner, M., &Serebryansky, E. P. (2018). Cadmium and atherosclerosis: A review of toxicological mechanisms and a meta-analysis of epidemiologic studies. Environmental Research, 162, 240-260. https://doi.org/10.1016/j.envres.2018.01.008

- Xie, S. (2024). Biosorption of heavy metal ions from contaminated wastewater: an eco-friendly approach. Green Chemistry Letters and Reviews. https://doi.org/10.1080/17518253.2024.2357213

- World Health Organization. (2017). Water quality and health - review of turbidity: Information for regulators and water suppliers. World Health Organization. https://iris.who.int/handle/10665/254631

- SK.Dhoke,Chemistry Teacher International; 5:283-290, (2023)http://dx.doi.org/10.1515/cti-2022-0052

- Afuye, G. G., Oloruntade, A. J., &Mogaji, K. O. (2015). Groundwater Quality Assessment in Akoko South East Area of Ondo State, Nigeria. *International Journal of Science and Technology, 5*(9). https://www.researchgate.net/publication/315840000_Groundwater_Quality_Assessment_in_Akoko_South_East_Area_of_Ondo_State_Nigeria

- Tran, N. T. K., Dinh, N. T., Tin, N. H., Roem, A. J., Schrama, J. W., &Verreth, J. A. J. (2016). Interaction between dissolved oxygen concentration and diet composition on growth, digestibility and intestinal health of Nile tilapia (Oreochromis niloticus). Aquaculture, 462, 101-108. https://doi.org/10.1016/j.aquaculture.2016.05.005

- Wu, S., Wu, S., Yi, Z., Zeng, F., Wu, W., Qiao, Y., Zhao, X., Cheng, X., & Tian, Y. (2018). Hydrogel-Based Fluorescent Dual pH and Oxygen Sensors Loaded in 96-Well Plates for High-Throughput Cell Metabolism Studies. Sensors, 18(2), 564. https://doi.org/10.3390/s18020564

- Khaled, B., Abdellah, A., Noureddine, D., Salim, H., &Sabeha, A. (2017). Modelling of biochemical oxygen demand from limited water quality variable by ANFIS using two partition methods. Water Quality Research Journal, 53(1), 24-40. https://doi.org/10.2166/wqrj.2017.015

- Li, J., Luo, G., He, L., Xu, J., & Lyu, J. (2018). Analytical Approaches for Determining Chemical Oxygen Demand in Water Bodies: A Review. Critical Reviews in Analytical Chemistry, 48(1), 47-65. https://doi.org/10.1080/10408347.2017.1370670

- Sharma, S., & Bhattacharya, A. (2017). Drinking water contamination and treatment techniques. Applied Water Science, 7, 1043-1067. https://doi.org/10.1007/s13201-016-0455-7

- Sankhla, M. S., Kumari, M., Nandan, M., Kumar, R., & Agrawal, P. (2016). Heavy Metals Contamination in Water and their Hazardous Effect on Human Health-A Review. *International Journal of Current Microbiology and Applied Sciences*, *5*(10), 759-766. https://doi.org/10.20546/ijcmas.2016.510.082

- Lavanya Khanna et.al., ANALYSIS OF HEAVY METALS CONTAMINATION IN WATER: A REVIEW, 2021 IJRAR December 2021, Volume 8, Issue 4.

- El Morabet, R., Barhazi, L., Bouhafa, S., Dahim, M. A., Khan, R. A., & Dahim, A. M. (2024). Water quality, heavy metal contamination and health risk assessment of surface water bodies of Mohammedia prefecture, Morocco. *Environmental Chemistry and Ecotoxicology*, *6*, 33-41. https://doi.org/10.1016/j.enceco.2023.12.002

- Chowdhury, S., Mazumder, M. A. J., Al-Attas, O., & Husain, T. (2016). Heavy metals in drinking water: Occurrences, implications, and future needs in developing countries. Science of The Total Environment, 569-570, 476-488. https://doi.org/10.1016/j.scitotenv.2016.06.166

- Patil, G., & Ahmad, I. (2011). HEAVY METALS CONTAMINATION ASSESMENT OF KANHARGAON DAM WATER NEAR CHHINDWARA CITY. *Acta Chimica and Pharmaceutica Indica*, *1*(1), 7-9. http://www.sadgurupublications.com/ContentPaper/2011/2_110_1(1)2011.pdf

- Gautam Patil* and Irfan Ahmad, (2023) Contamination of Heavy Metal in Water of Bhimgadh Dam at Seoni District Mp: Transporting, Toxicity and Treatment , International Journal of Chemical Sciences Research| Vol 19 Iss 7,