# Review Article

A Review of Unit Commitment and Economic Dispatch in Power Systems

.

ABSTRACT

In the current work, we study computational models and methods concerning Unit Commitment and Economic Dispatch (UCED), with a focus on Unit Commitment (UC) in centrally dispatched power systems. The optimization of UCED is an essential requirement for the reliability, cost-effectiveness, environmental sustainability, and energy justice of power systems. A total of 150 strictly peer-reviewed literature articles are examined. A group thereof is used to outline the historical evolution of UCED, computational tools, and related solvers. A subgroup of 62 review articles published since 2015 is used for a targeted evaluation based on 15 criteria. These include Demand Response (DR), flexibility parameters such as distributed resources optimization through Virtual Power Plants, and security-constrained UC parameters such as ramp up/down limits. Five categories of optimization methods, 22 models, and tools, as well as nature-inspired algorithms (NIAs), are appraised. This research adds a new dimension to the existing literature by including both CSOs and market participants’ interests under one view, while describing standardized prevailing UC solving methods and summarizing future requirements. Mainly Linear Programming (LP), Mixed Integer LP (MILP), and Mixed Integer non-LP (MINLP) remain dominant in real-system applications due to their robustness and compatibility with commercial solvers. Since 2019, reviews increasingly emphasize uncertainty management, flexibility, energy storage, policy implications as well as market participants’ deregulated and price-based environments. Deterministic UC models prevail, but stochastic and robust approaches are expanding. Realistic system applications on a variety of UCED-related issues, especially at (sub)hourly resolution, remain limited and are concentrated in the US, Brazil, and EU regions. Flexibility, energy storage, and uncertainty are common concerns for both CSOs and market participants. The existing modelling framework still needs to be advanced to transfer the application from IEEE systems to real systems while meeting uncertainty concerns. Three multiscale models are depicted for further examination.

Keywords: Unit commitment, Central dispatch, Power Systems, Optimization, Electricity markets

1. INTRODUCTION

The optimization of the Unit Commitment (UC) and Economic Dispatch (ED) problem, is a crucial issue, considering the necessity for the reliability, cost-effectiveness, environmental sustainability, and energy justice of power systems (Apon HJ,2024), (Singh A,2024), (Marzbani F,2024), (Goforth T,2025), (Kleanthis N, 2025). Both centralized and decentralized frameworks of markets have been studied since 1990 (Kaya A, 2025). The UC establishes the optimal timetable for power generation units, while the ED of load seeks to meet load demands while minimizing generation costs. Research on power system optimization can be categorized into short-term operational tasks and long-term financial planning. Short-term challenges primarily focus on operational and scheduling issues, such as real-time UCED, whereas long-term challenges emphasize aspects of power system design and planning. The main decisions required for a power system optimization problem, per timescale, are illustrated below [Fig.1].

Fig. 1. Decision-making in power system scheduling (modified from (Cho S., 2022))

The current work i) provides up-to-date evidence on how UCED optimization tools are shaped under constraints such as environmental ones; ii) examines only peer reviewed literature iii) focuses on the “current years” (2015-2026) following the inauguration of the Energy Union Strategy, and the establishment of the United Nations Sustainable Development Goals (SDGs) while giving an insight on the preceding time period, v) focuses on UC in centrally dispatch markets, while also examining the view of market participants. Whereas the UC problem, from the point of view of the CSO, is to cover market, policy, and security constraints such as ramp limits, market participants, i.e., generation companies (GENCO), are aiming at maximizing their profit. The current review mainly addresses UC aspects concerning CSOs, without neglecting interaction with the interests of GENCO/market participants. The optimization of UC can be formulated as a mixed-integer linear programming (MILP) or mixed-integer nonlinear programming (MINLP) problem due to the presence of both continuous (e.g., power output of units) and binary (e.g., on/off status of generators) decision variables. These methods commonly utilize solvers like Gurobi or CPLEX to handle the combinatorial aspects of UC by modelling the on/off statuses of generators and their operational constraints. Typical constraints can be such as power balance constraints, generator capacity limits, ramp-up and down rates, operating regulations, minimum up and downtimes, maintenance schedules, and environmental constraints. Optimization problems generally consist of an objective function (OF), decision variables, and constraints that the decision variables must satisfy. An OF for UCED (equation 1) typically minimizes the total cost over a given time horizon. Such costs may include: i) fixed Operation and Maintenance (O&M), ii) variable O&M, iii) fuel, iv) carbon dioxide emissions, v) start-up/ shut-down costs, vi) penalty for loss of load, vii) penalty for inadequate upward reserves.

t ϵ Τg ϵ G[Cgᶠⁱˣ · u₍g,t₎ + Cgᵛᵃʳ(P₍g,t₎) + Cgᶠᵘᵉˡ(P₍g,t₎) + Cgᶜᵒ₂(P₍g,t₎) + Cgˢᵗᵃʳᵗ · u₍g,t₎ᵘᵖ + Cgˢʰᵘᵗ · u₍g,t₎ᵈᵒʷⁿ] +t ϵ Τ [λₜˢʰᵉᵈ · LoadShedₜ + λₜʳᵉˢᵉʳᵛᵉ · ReserveShortfallₜ]

[1]

In the equation, C_g accounts for costs related to the generator, and C_t for costs related to penalties. The variable u_(g,t)  is a binary decision variable representing the on/off status of generator g at time t; it takes the value 1 if the generator is on and 0 if it is off. The variable P_(g,t) is a continuous variable denoting the power output of generator g at time t. To capture changes in generator status, u₍g,t₎ᵘᵖ and u₍g,t₎ᵈᵒʷⁿ  are binary variables indicating whether the generator g starts up or shuts down, respectively, at time t. The continuous variable LoadShed represents the amount of load not served (i.e., unmet demand) at time t, reflecting potential penalties for supply shortages. Similarly, ReserveShortfallₜ is a continuous variable measuring any shortfall in reserve capacity at time t, which may trigger additional costs or system reliability concerns.

The OF incorporates several critical constraints to guarantee the feasible and dependable operation of the power system. The power balance constraint ensures that, during each time period, the total power generated, along with any permissible load shedding, precisely satisfies the demand. Each generator's power output is limited by its minimum and maximum capacity thresholds, and any changes in output must adhere to ramp-up and ramp-down limits to prevent sudden fluctuations. The initiation and cessation of generators are regulated by logical constraints that connect the current and prior on/off statuses through binary indicators. Minimum up-time and down-time constraints ensure that once a generator is activated (or deactivated), it remains in that state for a specified minimum duration. To uphold system reliability, a reserve requirement constraint requires adequate spinning reserve capacity at all times, imposing penalties for reserve shortfalls when reserves are inadequate. Optionally, the model may impose a limit on total CO₂ emissions to adhere to environmental regulations. The binary characteristics of on/off, startup, and shutdown variables are strictly enforced, while power outputs, load shedding, and reserve shortfalls are treated as continuous and non-negative variables. Collectively, these constraints encapsulate the technical and operational realities of UC, enabling cost-effective and reliable scheduling of the power system.

A simplified OF for UCED optimization, on a simplified hypothetical power system, is described below [Fig. 2]. The system consists of 3 nodes, i.e., physical locations of production or consumption of electricity.

Fig.2. A Generic Objective Function of a simplified 3-node power system.

1.1 Optimization of power systems

Power system optimization addresses (Kaya A,2025), (Marcovecchio MG, 2014) (i) Power Capacity Expansion (PE), (ii) Economic dispatch (ED), and (iii) Unit Commitment (UC). Power Capacity Expansion (PE) deals with long-term energy generation planning, focusing on the projected capacity, type, and number of power units. Economic Dispatch (ED) concerns short-term decision-making, i.e., optimizing power plant operation to regulate the optimal power generated by individual units to balance load demand to a minimum cost and meet real-time demand. It involves the intensity at which power units should operate once they are running (“HOW MUCH”) while ALL units run to minimize the overall fuel cost of power units for a specific load demand. Unit Commitment (UC) concerns short-and medium-term decision-making. It concerns the optimal scheduling of on/off decisions and output levels of power units to minimize fuel costs. Decisions are made regarding when each unit remains on or offline, as well as how long the unit remains in that state. It specifies which units to switch on or off (“IF”) and when (“WHEN”). The ED process initially managed the operation of fossil fuel or nuclear power plants. Before 1980, several studies had outlined state-of-the-art methodologies for least-cost power system operation based on linear programming (LP) methods. In 1974, an ED process for using the Jacobian matrix was presented (Happ HH,1974). In a 1977 survey, the same author addressed the optimal load flow used by the industry (Happ HH,1977). A review in 1979 provided an outline of the essential analytical and computational topics involved in designing an LP method for rescheduling power system operation (Stott B., 1979). Between 1977 and 1988, the main approaches to ED were (Chowdhury BH,1990): i) optimal power flow (OPF), ii) ED in relation to automatic generation control (AGC), iii) dynamic dispatch, and iv) ED with non-conventional generation sources. By 1984, power generation dispatch tools were suitable for medium- to long-term scheduling or short-term views; however, with no more than one year at a time considered (Sterling MJH,1984). Following the necessity for clear investment scenarios imposed by VRE, greenhouse gas (GHG) emissions’ restrictions, storage planning, and Demand Response (DR), power optimization has advanced (Basu M, 2019), (Motta VN,2024). It evolved into more complex power planning processes. Such are the Multi-Objective Economic Emission Dispatch (MOEED) or Combined Economic Emission Dispatch (CEED) (Manikandan TR,2023). The necessity for a well-operating liberalized electricity market resulted in further optimization approaches, such as the coordinated Generation Expansion Planning (GEP) and Transmission Expansion planning (TEP), along with ED decision-making (Asgharian V,2022), (Gómez S,2024). The complexity of UCED depends primarily on the design of the electricity market of a specific region, i.e., whether the market is self-owned and centrally committed. It also depends on the type of power units, the type of fuel used, the type of demand covered (peak or basic), grid availability, and other uncertainty parameters. Such parameters can be short-term, e.g., demand behavior and the weather, and long-term, such as the future regulatory framework and level of economic growth (Gómez S,2024) (Jiang Y, 2024).

1.2 Problem statement and paper rationale

Given the aforesaid limitations, UCED optimization models need to adapt to increased variability and uncertainty and further technological and regulatory constraints. This means integrating stochastic elements and probabilistic forecasting to manage the unpredictable nature of renewable energy sources (RES). Moreover, policies aimed at reducing GHG emissions require models to account for both cost and environmental impact. In areas where electricity markets are becoming more competitive, UCED optimization models must adjust to new market rules, including bid-based dispatch and price caps, to ensure economic efficiency and reliability. Regional factors also affect optimization: areas with strong grid infrastructure can handle more VRE, while regions with weaker grids may need more conservative strategies and better reserve margins.

The global power system started being decentralized since the 1990s. However, a centralized structure has been maintained in some regions (Kaya A,2025), for which UC modelling becomes more challenging. The questions addressed in this work are (i) What methods are being advanced for solving the UCED problem, ii) How are optimization tools shaped under recent changing conditions, iii) How is UC optimization addressed in centrally dispatched markets.

The main advantage of a centralized dispatch market is that it guarantees the technical viability and, if possible, cost-effectiveness of the day-ahead dispatch (Ahlquist V, 2022).

Literature has been selected based on the following criteria: i) only peer -reviewed literature has been included as the most trustworthy source of information ii) emphasis was put on covering a diversity of journal sources iii) the main time period of investigation focuses on the “current years” (2015-2026) following the introduction of the United Nations Sustainable Development Goals (SDGs) on a global level which coincided with the inauguration of the European Union’s (EU) Energy Union Strategy (COM/2015/080) and subsequently the Regulation EU/2021/1119 and the “European Climate Law” (ECL) for achieving climate neutrality in the EU by 2050; iv) the inclusion of found literature in the current work was based on the presence of three specific set of terms as described in paragraph 2 Materials and Methods and terms related to the comparison criteria in Table 2.

1.2.1 A centrally dispatched electricity market

Electricity became a commodity in the framework of the liberalized market. Real-time market players balance supply and demand in several time intervals, less than 24 hours. The liquidity of electricity markets has evolved by shifting from intraday trading to intra-hour and continuous trading. In the EU, distinct wholesale electricity markets were designed: a) the Energy Derivatives Market ("Forward" market), b) the “Day-Ahead Market,” c) the “Intraday Market,” and d) the “Balancing Market.”

In a centrally dispatched electricity market, as in Greece, a Central System Operator (CSO) sets all facilities' power generation schedules to fulfill demand optimally. A centralized power system structure has been kept in some world regions (Kaya A,2025), (Silva WN, 2025). Examples of CSOs in the US are MISO, SPP, PJM, CAISO, NYISO, ISO NE, ERCOT in the Midcontinent, South West, North, California, New York, New England and Texas respectively. Additional examples are AEMO, ONS, CEN, IPTO, Grid-India and CENAC in Australia, Brazil, Chile, Greece, India and Mexico respectively. In order to maintain grid stability and balance supply and demand throughout the centralized market, the central operator supervises the process, from initial scheduling and reserve procurement to real-time modifications.

The EU’s "third Energy Package" set the foundation for transforming the Greek electricity market by promoting competition, removing market distortions, and modernizing network infrastructure. Today, the Greek electricity market is shaped by Law 4425/2016 (Official Gazette A’185/30.09.2016), as amended by Law 4512/2018 (Official Gazette A'5/17.01.2018). This legislation formed four markets based on EU Regulation 1227/2011 and Law 4389/2016 (OJ Α' 94/27.05.2016) concerning quarterly electricity forward products' auctions, known as the "NOME law," which was later discontinued in 2019. This background introduced the new Hellenic Power Exchange (HPE) and the Hellenic Energy Exchange (HEnEx). HEnEx oversees the operation of the Forward, Day-Ahead, and Intra-Day Markets, while the national Independent Transmission System Operator (IPTO) manages the Balancing Market. HPE acts as the Nominated Electricity Market Operator (NEMO), providing marketplaces, trading venues, and platforms, ensuring efficient price development for the Greek Bidding Zone, maintaining secure and reliable energy trading in Southeast Europe. The Greek Balancing Market operates on a central dispatch/unit model, where production, consumption, and dispatching plans are governed by the following IPTO procedures:

i) IPTO I: Balancing Capacity Market: Implemented through the Integrated Scheduling Process (ISP), it aims to secure the required Balancing Capacity in advance and achieve a schedule that meets the technical constraints of the Transmission System and the Balancing Service Entities.

ii) IPTO II: Balancing Energy Market: Quantities and prices are determined for the activation of Balancing Energy by respective Service Providers to balance energy supply and de-mand, considering market schedules and the real-time state of the Transmission System.

iii) IPTO III: Balancing Market Settlement: Calculation of Balancing Energy and Balancing Capacity quantities and the monetary value of participants' debits and credits, as well as the debits and credits to the Balance Responsible Party for imbalances. Greece shifted its Market Time Unit (MTU) within its bidding zone from an intra-hour to a 15-minute interval in October 2025 in accordance with the regulation EU/2019/943 on the EU’s internal electricity market.

1.2.2 Deregulated electricity market

In centralized electricity markets, costs need to be minimized while demand has to be covered in a reliable manner. In deregulated markets, the production, transmission and retail services operate separately. Typical operations in a deregulated energy market are the price-based or profit-based unit commitment (PBUC) conducted by the market participants, which aim at maximization of profit, and the market based economic dispatch (MBED) by an Independent System Operator (ISO) (Amudha A, 2013), (Montero L, 2022), (Gupta R, 2023), (Lejeune MA, 2024), (Lotfi H, 2024).

2. material and methods

A literature review on UCED methods and tools was conducted for early years (prior to 2015) and current years (2015- 2026). The review was made in two phases: i) searching in a basic database (ScienceDirect) using three sets of specific search terms, “a,” “b,” and “c,” and additional terms, such as “flexibility” related to the 15 comparison criteria in Table 2; ii) quests on further databases for solely current years on the same terms. The specific terms studied within “advanced search” and “title abstract-author-specified keywords” were a) “economic dispatch,” power; b) “unit commitment,” power; c) “unit commitment”, “dispatch”.

Initially, depicted energy and power models and tools are outlined in paragraph 3.1, and a categorization of power system optimization methods is shown in paragraph 3.2. After reviewing literature based on topic grouping, results were derived in narrative and tabular form in the third chapter. Bibliometrics on the basic database were derived for “current years” (2015-2026) and for three sets of search terms, including OS. The advantages and disadvantages of UC prevailing solving methods are shown in Table 1. An overview of 62 review articles linked to UCED is shown in Table 2. An overview of publications tested under realistic conditions on a variety of UCED-related aspects is given in Table 3, whereas a further comparison of publications based on modelling type is presented. Models, to be further developed at the country or regional level, are described in paragraph 3.6. Results, discussion, and conclusions are stated in chapters 4 and 5.

3. Optimization techniques, methods, and tools for the UCED problem

3.1 Current status

There is a vast spectrum of computational tools available, from small-scale to worldwide and long-term power scheduling. Reviews proving this include i) a review of 75 existing energy and electricity tools (Ringkjøb HK, 2018), ii) a review of 10 integrated energy national models in the Netherlands (Fattachi A, 2020), iii) a survey on integrated energy systems (IES) on 22 tools identified as “suitable for sustainable development” (Song D, 2022), and iv) power market models for the clean energy transition (Haugen M, 2024). Restrictions of distinct models can be addressed with “model linking.” Model linking can combine long-term models and short-term models, e.g., PE and production cost models. For specifically solving the UCED problem, Algebraic Modelling Languages (AML) such as the General Algebraic Modelling System (GAMS) and Python are advantageous. Though they do not directly solve a problem, they offer links to state-of-the-art algorithms (solvers). The tools, built with AMLs, provide optimization set-tings (e.g., for cost minimization) and handle constraints that call for solvers. A group of 22 common energy and power models and tools, a third of which are Open Source (OS), have been selected based on their observed frequency of occurrence in the literature. They are categorized into long-term energy system planning and optimization models, short-term power system operation models, and multiscale and flexible energy models. Some typical long-term models are MARKAL (Market Allocation Model) (Ringkjøb  HK, 2018), (Song D, 2022) (Victor N, 2024), TIMES (Integrated MARKAL-EFOM System) (Silva WN,2025), (Luxembourg SL, 2025), ELIN (Electricity Investment Model) (Goop J, 2021), EnergyPLAN (Advanced Analysis of Smart Energy Systems) (Ringkjøb  HK, 2018), (Lund H, 2021), BALMOREL* (Partial equilibrium model) (Ringkjøb  HK, 2018), (Wiese F, 2018), NEMO Next Energy Model system for Optimization (Ringkjøb  HK, 2018), (Handayani K, 2023), TEMOA* (Tools for Energy Mod. Optimization or Analysis) (Ringkjøb  HK, 2018) (Song D, 2024), (Nicoli M, 2022), POLES (Prospective Outlook on Long term Energy Systems) (Ringkjøb  HK, 2018), (Després J., 2016) and PRIMES (Price-Induced Market Equilibrium System) (Ringkjøb  HK, 2018), (Capros P, 2018). Proven short term models are e.g. ODIN (Open Dispatch Model for the Nordic Power System) (Nycander E, 2022), PowNet (Network-Constrained UC /ED Model), (Majidi H, 2025) EPOD (EU Power Dispatch Model), WASP (Wien Automatic System Planning) (Zeljko M,2020), DISPA-SET* (UC / Power Dispatch Model) (Katsanevakis A,2022), (Groissböck M, 2019), MATPOWER* (Programming package), (Groissböck M, 2019), (Majidi H, 2025), PyPSA* (Python for Power Systems Analysis) (Ringkjøb HK, 2018), (Unnewehr JF, 2022), (Groissböck M, 2019), (Majidi H, 2025) and ELMOD* (Deterministic linear/mixed-integer dispatch model), (Majidi H, 2025), (Groissböck M, 2019). Finally, MultiScale and Flexible proven models are the Flex-Tool* (Least-cost optimization tool) (Majanne Y, 2022) OSeMOSYS* - Global energy systems optimization model), (Ringkjøb  HK, 2018), (Groissböck M, 2019), Pyomo* (Python Optimization Mod. Objects software), (Hart WE, 2012), Calliope* (Multi-scale energy systems modelling) (Ringkjøb  HK, 2018), (Song D, 2024), (Pickering B, 2022), (Groissböck M, 2019), (Majidi H, 2025), and PLEXOS® (Integrated Energy Model) (Goforth T, 2025), (Ringkjøb  HK, 2018), (Hamdi M, 2024).

3.2 Power Systems Optimization Μethods

Several categories of methods have been developed to solve the optimization problem of power systems. A generic categorization of available techniques, methods, and tools into five categories is the following: i) Traditional techniques: Linear programming (LP), Non-linear programming (NLP), Mixed-Integer Programming (MIP), Quadratic Programming (QP), (Pickering B, 2022), (Gandhi O, 2016)  ii) Nature – inspired Algorithms (NIAs): Evolutionary Algorithms (EAs): Heuristic search methods based on Darwinian evolution, such as bio-inspired algorithms (Sulaiman MH, 2020) Bacteria Foraging Optimization (BFO) (Pandit N, 2012) and Particle Swarm Optimization (PSO): A swarm based meta-heuristic optimization algorithm, found in variated applications, such as in a Grey Wolf Optimization and PSO (GWO-PSO) (Shaheen AM, 2021); iii) Artificial intelligence (AI) techniques: Artificial Neural Networks (ANN), Genetic Algorithm (GA) and Machine Learning (ML) methods, (Basak S, 2022), (Asif M, 2024) as well as additional approaches (Ahmed I, 2026) iv) Hybrid Artificial Intelligent Algorithms: Examples include PSO and Artificial Fish Swarm Algorithm (ASFA) (Yuan G, 2019) and v) Quantum Computing (QC) Methods: These methods might show some prospects (Morstyn T, 2024).

3.3 Open Source Modelling and Open Source Data

Open Source (OS) modelling tools and open energy data emerged in the early 2010s. There are five features of openness (Weibezahn J, 2019), i.e., open data, an OS modelling language, OS model code, OS solvers, and open-access publications. In 2019, Groissböck (Groissböck M, 2019) concluded that OS models such as TEMOA, pyPSA, and OSeMOSYS were mature for reliable application. Yet, the legal status of the authorization for the redistribution of input data is a common problem (Pfenninger S, 2018). Many OS models, despite having OS code, do not have entirely open data, making their use difficult (Capros P, 2018). During a scenario-based model comparison with five OS models (Van Ouwerkerk, 2022), it was concluded that “stronger collaboration and exchange of data between scientists could substantially improve future model comparisons.” Data papers are a result of data-driven open science and support the FAIR principles (Findability, Accessibility, Interoperability, and Reusability) for research data management. Publishing “data papers” provides evidence while adding to the creation of knowledge and data per se (Schöpfel J, 2019). A typical example is the data paper by Guerra et al. (Guerra K, 2022), accompanying the respective research paper (Guerra K, 2022b).

3.4 Evidence from review and research papers

Evidence for early years is described in paragraph 3.4.1, while conclusions for current years are in paragraph 3.4.2. In comparison to similar reviewing work, the current work covers solely peer-reviewed literature, covering a substantial time span. Pros and cons of UC prevailing solving methods are shown in Table 1, whereas an overview of review articles linked to UCED is summarized in Table 2, in paragraph 3.4.3. The 62 review articles from current years are analyzed based on 15 criteria/characteristics. Those were categorized into parameters 1 to 7, i.e., parameters which concern CSOs in central dispatch markets 1:OPF, 2:Smart Grid, 3:DR,4: Electricity Market and policy implications, 5:Security Constraints (SC/SCUC),6:GEP/TEP/transmission management, 7:Short-term Scheduling; 8:Flexibility, 9:Energy Storage, 10:Uncertainty management modelling, 11:Electric Vehicles (EVs), 12:CEED, 13:AI 14:QC, and lastly the 15th parameter of Profit Maximization (UC/PBUC), which is a challenge for GENCO/market participants.  The 8th to 14th parameters are both a concern of CSOs and GENCO/market participants, whereas the 15th parameter of PBUC is a concern of GENCO/market participants.

3.4.1 Early years’ evidence

Review papers published before 2015 addressed the risk and variability of weather-dependent RES (wind and solar) in ED and UC (Ren B, 2009) (Foley AM, 2012) (Inman RH, 2013). The GA has been used for the solution of UC (Swarup KS, 2003). The PSO method was reviewed (Mahor A, 2009), and plug-in EVs were discussed (Peng M, 2012). By 2013, the UC problem targeted profit maximization (PBUC) without the obligation to cover the demand using a Memory Management Algorithm (MMA) (Amudha A, 2013). By 2014, the means for mitigating the adverse impacts of transmission congestion, VRE inclusion, and security constraints on the electricity market had been extensively addressed in the literature. Transmission management has been studied through a Shift Factor Optimal Power Flow (SF-OPF) model (Nappu M, 2014), RES inclusion through MILP short-term scheduling models of insular power systems (Simoglou CK, 2014), and the multi-objective security-constrained UC (SCUC) problem has been considered through MILP (Norouzi MR, 2014) including dynamic ramp limits and (Karami M, 2013). Review papers in 2015 addressed AI-based load demand forecasting techniques for smart grid and buildings (Raza MQ, 2015) and optimal power dispatch in a wind farm (Zhang J, 2015).

3.4.2 Current year’s evidence

The issues addressed still refer to the risk and variability of RES, yet they also initiate further discussions, which are categorized in paragraphs 3.4.2.1 to 3.4.2.6. Current year’s evidence is outlined analytically in Table 2 for all 62 examined publications. From those, review articles in the basic database with high impact, concern, photovoltaic (PV) power forecasting (Antonanzas J, 2016), and the integration of EVs in smart grids (Tan KM, 2016) (Ahmad T,2020). Further review articles with high impact address the topics of UC in the presence of VRE, e.g. with forecasting (Abujarad SY, 2017), (Ahmed, 2019) storage impacts on UC; two-stage robust optimization (Qiu H,2022), RES power systems and Combined Heat and Power (CHP) production (Nazari-Heris M,2018) (Bagherian MA,2020) microgrid energy management systems (Thirunavukkarasu GS, 2022), integrated energy systems (Song D,2022), GEP (Koltsaklis NE, 2018), AC OPF (Capitanescu F., 2016); ED in multi-area power system (MAELD) (Kunya AB,2023) (Sharifian Y,2024) QC (Liu H,2023) (Golestan, 2023), global optimization of economic load dispatch in large scale power systems (Hassan MH, 2024), distributed optimization (Wang Y, 2017), heuristic algorithms for optimal combined CHP dispatch; Short-term load forecasting (STLF) (Kong J, 2020) (Eren Y,2024) and OS optimization tools (Groissböck M, 2019). More recent work covers topics on UC and scheduling in islands with VRE (Psarros GN, 2023), QC, MAELD (Sharifian Y, 2023), and hybrid AC-DC microgrid control strategies (Dahane AS,2024). It also addresses models for operation with DR and deep learning approaches for STLF and ED in large-scale power systems (Eren Y, 2024). Frequency-constrained UC (Olasoji AO, 2024), Metaheuristic optimization (Chicco G, 2020), PBUC (Abdi H, 2021), cost recovery in developing countries (lychettira KK,2021), and modelling in deregulated electricity markets are addressed (Chimnoy L, 2019). Moreover, what is tackled are advances in stochastic UC (Håberg M, 2019) (Scuzziato MR, 2021), and hydropower plant UC (Vieira DAG, 2022), optimal dispatch in wind farms, plug-in EVs in a microgrid (Moghaddas TSM, 2016), non-fuel-based mechanisms (Taylor JA, 2016), short-term scheduling of hydropower plants (Nazari-Herris M, 2017), reliability impacts of flexible ramp products (Cornelius A, 2018), transmission management, distributed optimization approaches, supply- and demand- side optimal load scheduling in smart grids, (Lu X, 2018) (Maneesha A,2021), the implication of production tax credit on ED (Liu J,2022), operations research in OPF (Skolfield JK,2022), uncertainty management and security constrained UC (Aharwar A, 2023), tools for net-zero emission energy systems (Samarasinghe SL, 2025)  BESS within UC (Olivos C, 2023), Combined Heat and Power based UC, (Abdi H, 2023) OPF with UC and AI optimization for power system applications (Krishnamurthy S, 2026) .

3.4.2.1 Prevailing methods for solving the UC problem.

The categorization of power system optimization methods in paragraph 3.2 leads to five categories of methods. Whereas traditional techniques show limitations in power system optimization solving, some metaheuristic algorithms, inspired by the second category, i.e., NIAs, are highly efficient (Mandal PK, 2023). Such are the cut and branch (CaB) method (a non-linear mixed-integer optimization problem), the “Fire Works Algorithm” (FWA), the “Coyote Optimization” (COA) algorithm, the “Jaya Algorithm” (JA), and other Nature-inspired optimization techniques (Zheng H, 2016) (Saravan B, 2016) (Ali ES, 2023) (Chaudhary V, 2023) (Pandit M, 2020). A review on metaheuristics (MHs) (Valencia-Rivera GH, 2024) concluded that the most used MHs in electric power systems are, in order of usage recurrence, PSO, GWO, GA, Cuckoo Search (CS), and Differential Evolution (DE). The ad-vantages and disadvantages of prevailing UC solving methods are shown below (Table 1). The presented algorithms are used for the UC optimization problem, even though some entries can also be applied to ED. They are categorized into Swarm Intelligence (SI), and Evolutionary Algorithms (EAs).

Table 1. Pros and cons of UC prevailing solving methods

| Algorithm | Advantages | Disadvantages |
| --- | --- | --- |
| DE – PSO (SI), (Pandit M, 2020) (Valencia-Rivera GH, 2024) (Montero L, 2022) | Flexible and effective for a variety of problems. | Slow convergence, computational intensity, premature convergence. |
| GWO (SI), (Mandal PK, 2023) (Pandit, 2020) (Valencia-Rivera GH, 2024) | Fewer parameters, basic ideas, simple application. | Slow convergence rate, poor solution accuracy. Tendency for local optimum. |
| JA (SI) (Chaudhary V., 2023) (Venkatasatish R 2023), (Zeng L, 2023) | Simplicity – Parameter-free nature, easy to implement. | Premature convergence. |
| COA (SI) (Ali ES, 2023) , (Abu Zitar R, 2022) (Feng Y, 2024) | Balance between exploration and exploitation. | Requires further empirical validation for UC problems. |
| FWA (SI) (Saravanan B., 2016), (Montero L, 2022),(Samarasinghe SL, 2025) | Robust exploration capabilities in optimization tasks. | Challenges such as slow convergence speed and low accuracy. |
| CaB (SI) (Zheng H, 2018) ,(Valencia-Rivera GH, 2024) (Rateele TN, 2023) | Effective for complex constraints, optimal solutions. | Can be computationally intensive for large-scale systems. |
| GA (SI) (Pandit M., 2020),(Valencia-Rivera GH, 2024) (Sheng W, 2023) | Flexible and effective for a variety of problems | Slow convergence, computational intensity, and premature convergence |
| CS (Eas) (Mandal PK, 2023),(Valencia-Rivera GH, 2024) | Effective for multimodal problems (few parameters). | Slow and premature convergence. |
| GA (Eas) (Pandit M, 2020), (Valencia-Rivera GH, 2024) (Sheng W, 2023) | Flexible and effective for a variety of problems | Slow convergence, computational intensity, and premature convergence |

3.4.2.2. Short-term VRE forecasting is essential for reliable UCED solving.

PV and wind power production forecasts have been widely stated, as in Sheng et al. (Sheng W, 2023) and Rivera et al. (Valencia-Rivera GH, 2024). Methods to deal with VRE uncertainty concern stochastic optimization (SO), robust optimization (RO), and interval optimization (IO). In the case of the IO scheduling model, VRE outputs are expressed as empirical prediction intervals (Zeng L, 2023). Short-term VRE forecasts on an hour-scale were studied in research by Feng et al. (Feng Y, 2024) during a four-hour ultra-short-term power forecast. Methods for short-term uncertainty of VRE were studied in the review by Samarasinghe et al. in 2025 (Samarasinghe SL, 2025). Also, the necessity for modelling ancillary services (AS) was talked about in literature, as in (Olasoji AO, 2024), as crucial for the safety, security, and dependability of power systems.

3.4.2.3 Provision has to be made for excess locally generated energy.

Storage options are needed to guarantee the reliability of the energy system and the optimal operation of the energy market (Rateele TN, 2023), (Dimitriadis CN, 2023). A review of integrating Battery Energy Storage Systems (BESS) in UC in the day-ahead market (Olivos C, 2023) showed that BESS is a research area that can strengthen sustainable, resilient, and efficient energy systems. A bibliometric analysis on the optimization of energy storage systems’ (ESS) size (Tahir H,2024) showed that including load variations in the design process could enhance relevant research. Multi-objective optimization of thermal dispatch scheduling, with differently matching load levels, was also examined. The Non-dominated Sorting GA-III obtained optimal dispatch, considering base, intermediary, and peak load levels (Hammerschmitt BK, 2024).

3.4.2.4. Emergencies require fast, real-time optimization.

A simple learning-based method for the Stochastic UC (SUC) problem has been proposed (Liu X, 2023) where the SUC and ED problems are solved with CPLEX and JuMP solvers. The model was tested on an Illinois 200-bus system (Approximation of a power system with 200 substations). Highly reduced solution times were reported. In 2024, Li P. et al. (Li P, 2024) presented a single-period optimization model combining alternating current OPF (AC OPF) and UC variables, which restored the capability of Security Constrained ED (SCED). It was applied to the IEEE 118-bus system, the IEEE 300-bus system, the Polish 2383-bus system, and a 13977-bus system. Also, the contribution of pumped storage units and energy storage was tackled with difficulty in real-time scheduling (Chen K, 2023).

3.4.2.5. Case study evidence strengthens the credibility of UCED solving.

Several case studies are needed for a fair comparison of solutions (Abdi H, 2023). Even then, assumptions, limitations, hardware, and software data need to be alike. The majority of studies focus on the evaluation of optimization using only one case study (Marzbani F, 2024).

3.4.2.6. GENCO’s profit-based UC

UCED modelling is also perceived as Production Cost Modelling (PCM) from GENCO’s point of view and incorporates energy justice and equity considerations in power optimization (Goforth T, 2025), (Lejeune MA, 2024). Profit-based UC (PBUC) is gaining ground (Abdi H, 2021) (Lejeune MA, 2024) (Lotfi H, 2024). Additional parameters that are of interest for GENCO/market participants are power system flexibility, energy storage, uncertainties, and management (Antonanzas J, 2016), (Tan KM, 2016), (Abujarad SY, 2017), (Chimnoy L, 2019), (lychettira KK, 2021).

3.4.3 Overview of review articles.

An examination of 62 reviews publicized in recent years in Table 2, is based on 15 basic characteristics, sorted into three groups as stated in paragraph 3.4. Publications in the first group (1st to 7th parameter) are a typical concern of CSOs. Publications within the second group (8th to 14th parameter) tackle both the concerns of CSOs and market participants and rose in number compared to other parameters since 2019. PBUC appears as a tackled issue in review publications, since 2019. All examined publications on PBUC, which is mainly the concern of the GENCO/market participants, also tackled the parameter of uncertainty modelling, but also typical CSO concerns such as DR and SCUC in the most recent years following 2022. It can be indicated that the concerns of CSOs and market participants on flexibility, energy storage, and uncertainty management gradually grow in parallel for CSOs and market participants. In addition, environmental and energy policy implications and uncertainty management stand out as parameters of concern throughout all examined years.

Table 2. Overview of review articles

| YEAR | REFFERENCE | OPF | Smart Grid | DR | Electricity Market & Policy Implications | SC/ SCUC | GEP /TEP /transm. Mgmt | Short-term scheduling | Flexibility | Energy Storage | Uncertainty mgmt/ modelling | EV | CEED | AI | QC | PBUC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2015 | Raza MQ |  | X |  |  |  |  |  |  |  |  |  |  | X |  |  |
| 2015 | Zhang J |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |
| 2016 | Gandhi O |  | X |  |  |  |  |  |  | X | X | X |  | X |  |  |
| 2016 | Antonanzas J |  |  |  | X |  |  | X | X |  |  |  |  |  |  |  |
| 2016 | Tan KM |  |  |  |  |  |  |  | X |  |  | X |  |  |  |  |
| 2016 | Capitanescu F. | X |  |  |  | X |  |  |  |  | X |  |  |  |  |  |
| 2016 | Moghaddas TSM |  |  |  | X |  |  |  |  |  | X | X |  |  |  |  |
| 2016 | Taylor JA |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |
| 2017 | Abujarad SY |  |  |  |  |  | X |  | X | X |  | X |  |  |  |  |
| 2017 | Nazari-Herris M, 2017 |  |  |  |  | X |  | X |  |  |  |  |  |  |  |  |
| 2017 | Wang Y | X |  | X |  | X | X |  |  |  |  |  |  |  |  |  |
| 2018 | Ringkjøb HK | X |  | X | X |  |  |  | X | X | X |  |  |  |  |  |
| 2018 | Nazari-Herris M,2018 |  |  |  |  |  | X |  |  |  |  |  |  | X |  |  |
| 2018 | Koltsaklis NE |  |  | X |  |  | X |  |  | X |  | X |  |  |  |  |
| 2019 | Groissböck M | X |  |  |  |  |  |  |  |  | X |  |  |  |  |  |
| 2019 | Ahmed A |  |  |  | X |  |  |  |  | X | X |  |  |  |  | X |
| 2019 | Håberg M |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |
| 2019 | Chimnoy L |  |  |  | X |  |  |  | X |  | X |  |  |  |  |  |
| 2020 | Fattahi A |  |  | X |  |  |  |  | X | X | X |  |  |  |  |  |
| 2020 | Kong J |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |
| 2020 | Ahmad T |  | X |  |  |  |  |  |  |  | X |  |  |  |  |  |
| 2020 | Bagherian MA |  |  |  |  | X |  |  |  | X |  |  |  |  |  |  |
| 2020 | Chicco G |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2021 | Scuzziato MR |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |
| 2021 | Abdi H |  |  |  |  |  |  |  |  |  | X |  |  | X |  | X |
| 2021 | Maneesha A | X | X | X |  |  |  |  |  |  |  | X |  |  |  |  |
| 2021 | lychettira KK |  |  | X | X |  |  |  | X |  |  |  |  |  |  |  |
| 2022 | Cho S | X |  |  | X |  | X |  |  | X | X |  |  | X |  |  |
| 2022 | Ahlqvist V |  |  | X | X |  |  |  | X | X |  |  |  |  |  |  |
| 2022 | Song D | X |  |  |  |  |  |  |  | X | X |  |  | X |  |  |
| 2022 | Qiu H |  |  |  |  |  |  |  | X |  | X |  |  |  |  |  |
| 2022 | Thirunavukkarasu GS | X |  | X |  |  |  |  | X |  | X | X | X |  |  |  |
| 2022 | Liu J |  |  |  | X |  |  |  |  |  | X |  |  |  |  |  |
| 2022 | Vieira DAG |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| 2022 | Skolfield JK | X | X |  | X |  | X |  |  |  |  |  |  |  |  |  |
| 2022 | Montero L |  |  |  |  |  |  |  |  |  | X |  |  | X |  | X |
| 2023 | Kunya AB |  | X | X |  |  |  | X |  |  |  | X |  |  |  |  |
| 2023 | Golestan S |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |
| 2023 | Abdi H |  |  | X | X | X |  |  | X | X | X |  |  | X |  | X |
| 2023 | Psarros GN |  |  | X | X |  |  | X |  | X | X |  |  |  |  |  |
| 2023 | Aharwar A |  |  |  |  | X |  | X |  | X |  |  |  |  |  | X |
| 2023 | Liu H | X |  |  |  |  |  |  |  |  |  |  |  |  | X |  |
| 2023 | Mandal PK |  | X |  |  |  |  |  |  |  |  |  |  | X |  |  |
| 2023 | Olivos C |  |  |  | X |  | X |  |  | X |  | X |  |  |  |  |
| 2023 | Chen K |  | X | X |  |  |  | X | X |  | X |  |  |  |  | X |
| 2024 | Sharifian Y |  |  |  |  |  |  |  |  |  | X | X | X |  |  |  |
| 2024 | Marzbani F |  |  |  | X |  |  |  |  |  | X |  |  |  |  |  |
| 2024 | Motta VN | X |  | X |  |  | X |  |  |  | X |  |  |  |  |  |
| 2024 | Gómez S |  |  |  |  |  | X |  |  |  | X |  |  |  |  | X |
| 2024 | Haugen M |  |  | X | X |  |  |  | X | X | X |  |  |  |  |  |
| 2024 | Morstyn T | X |  |  | X |  |  |  | X |  |  |  |  |  | X |  |
| 2024 | Eren Y |  |  | X | X |  |  |  |  |  | X |  |  |  |  |  |
| 2024 | Dahane AS |  | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| 2024 | Hassan MH |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |
| 2024 | Olasoj AO |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |
| 2024 | Valencia-Rivera GH | X | X | X | X |  | X |  |  |  |  |  |  | X |  |  |
| 2024 | Tahir H |  |  | X |  |  |  |  |  | X | X | X |  |  |  |  |
| 2025 | Goforth T |  |  |  | X |  | X |  |  |  |  |  |  |  |  |  |
| 2025 | Kaya A | X |  |  | X | X | X | X |  |  | X |  |  |  |  |  |
| 2025 | Samarasinghe SL |  |  |  | X |  |  |  |  |  | X |  | X |  |  |  |
| 2025 | Plazas-Niño F |  |  | X | X |  | X | X | X |  | X | X |  |  |  |  |
| 2026 | Krishnamurthy S | X |  |  |  |  |  |  |  |  |  |  |  | X |  |  |

3.5. Modelling and energy policy.

The integration of UCED models into energy policy frameworks has played a fundamental role in shaping both operational strategies and long-term investment decisions within modern power systems. These models are central to electricity market operation and planning, and their outputs have direct implications for the formulation of market regulations, capacity mechanisms, carbon pricing strategies, carbon neutrality, and VRE integration policies (Goforth T, 2025) (Kleanthis N., 2025) (Gómez S, 2024), (Hamdi M, 2024). One of the most significant impacts of UCED modelling on energy policy lies in its alignment with the merit-order principle (Cebulla F, 2017). The increasing penetration of VRE, which has near-zero marginal costs, has caused a downward shift in the merit-order curve. This reduces wholesale electricity prices and displaces conventional fossil-fuel generators, a dynamic well-documented in European markets where studies have shown that each additional GWh of wind power could reduce marginal prices by approximately €4.54 per MWh (Pluta M,2020). While beneficial for consumers, this also leads to declining revenues for dispatchable generators, giving rise to the “missing money” problem, where market prices are insufficient to sustain investment in reliability-critical infrastructure. UCED modelling has consistently identified this challenge, prompting policy responses such as capacity remuneration mechanisms. For example, Poland’s 2017 introduction of a capacity market was supported by simulations demonstrating insufficient revenue in energy-only markets (Pluta M, 2020). Models on UCED also play a vital role in assessing the environmental effectiveness of market tools such as carbon pricing and emissions trading schemes (Shen X, 2025). By incorporating carbon costs into dispatch algorithms, researchers can simulate how market behavior shifts in response to policy. Moreover, AI-based optimization for power system applications (Krishnamurthy S, 2026) (Ahmed I, 2026) appeared in the literature, especially following 2022, whereas the Combined Economic Emission Dispatch (CEED) is mentioned in several cases, as in (Mahdi FP, 2018) and (Manikandan TR,2023). As power systems grow more complex and variable—driven by high shares of VRE, demand-side participation, and decentralized generation—UCED models have evolved to incorporate uncertainty and system flexibility. Robust, stochastic, and uncertainty-constrained formulations now allow for more accurate simulation of real-world conditions, including load variability, weather disruptions, and contingency constraints (Sharifian Y, 2024) (Lejeune MA, 2024) (Cebulla F, 2017). These advanced models have been instrumental in shaping policies related to reserve procurement, demand-response program design, storage deployment, and the evaluation of real-time balancing markets. In addition, the management of distributed energy resources (DER), i.e. small-scale power generation can be enhanced through Virtual Power Plants that act as aggregators that coordinate the operation of many power sources (Gholami K, 2025). VPPs can provide significant flexibility by combining a variety of DERs, including VRE, battery storage, EVs, and DR resources. On the other hand, VPPs face the challenge of uncertainty management that occurs from e.g. peak energy demand.

3.6. UC modelling on the country and regional level.

Principal types of constraints to be taken into consideration during UC modelling are i) generator constraints (generation limits, ramp-up and down limits, minimum up and downtime); ii) VRE limitations; iii) available energy storage technological options; iv) The supply -demand balance; v) reserve constraints; and vi) transmission constraints. Additional relevant factors are the clustering of similar generators to increase flexibility of the system and tackling uncertainty. Though UC can undergo simplifications in order to be solved in shorter time periods, the solution quality is affected significantly if ramp up and down limits, adequate available reserves and transmission limits are not taken properly into consideration (Wuijts RH, 2024).

Optimization involves the choice of one or more reliable tool(s), the choice of an appropriate “optimization solver,” and the appropriate combination of the two. Previous research has shown that soft linking between long-term energy models and short-term models is commonly used (Kleanthis N, 2025) (Ringkjøb HK, 2018). The authors depicted three models outlined in paragraphs 3.1 and 3.2 as suitable for further elaboration on a country or regional level, based on their occurrence and appearance in recent literature. Those are the multiscale models PLEXOS®, (Hamdi M, 2024) (Samarasinghe SL, 2025) OSeMOSYS, and FlexTool, (Kleanthis N, 2025) (Majanne Y, 2022) (Niet T, 2021) (Plazas-Niño F, 2025).

Widespread used, reliable solvers are (Anand R, 2017): i) Gurobi Optimizer, for solving LP, QP, MILP, and mixed-integer quadratic programming (MIQP) problems; ii) IBM ILOG CPLEX Optimizer for large-scale optimization, supporting LP, integer programming, and QP; iii) FICO Xpress Optimizer, supporting algorithms such as primal and dual simplex; iv) MOSEK for MISOCP; and v) BARON for MINLP (Montero L,2022). Modelling on an hourly basis on IEEE test systems is apparent in the literature in deterministic case studies since 2014 (Marcovecchio MG, 2014), (Chowdhury AFMK, 2020), (Castelli AF, 2024) (Plazas-Niño F, 2025), as well as on stochastic case studies, such as in the case of (Stott B, 2023). Modelling on other time scales can be found in (Singh A,2024), (Manikandan TR, 2023) (Hart WE, 2012). IEEE Systems are mentioned in the literature, (Weibezahn J, 2019), (Van Ouwerkerk J, 2022) (Foley AM, 2012) (Inman RH, 2013) (Mahor A, 2009) extensively on IEEE bus 14-, 30-, 40-, 118-, 140-unit systems and IEEE Reliability Test Systems (RTS) 81,84,95,141,144. On a multi-year time horizon, real systems tested with long-term Energy System Optimization Models (ESOM) concern typically (Plazas-Niño F, 2025) on a global level, the application on the EU level with POLES in 2017 (Després J, 2016), PRIMES in 2018 (Capros P, 2018), TIMES in 2024 (Luxembourg SL, 2025), and PLEXOS (Hamdi M, 2024) in 2024 on the Egyptian power system.

Table 3. Overview of publications tested under realistic conditions on a (sub)hourly scale

| REF & YEAR | ISO/RTO or Real System | Country or Region | Minimum Time Horizon | Model Type | Problem Type | Grid Modelling | Generation Mix Used |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Cebulla F, 2017 | ENTSO-E | Germany | Hour | Merit-order vs UC | MILP | Simplified grid | Thermal, RES |
| Cornelius A, 2018 | MISO | US | 10 minutes | Ramp product market study | LP | Nodal Direct Current (DC) | Thermal, Wind |
| Chowdhury AFMK, 2020 | Real utility-scale systems | Cambodia | Hour | PowNet. Network-constrained UCED | MILP | Nodal (DC) Network | Thermal, Hydro, RES |
| Pluta M, 2020 | Polish Power System | Poland | Hour | ED & UC for System Analysis (MEDUSA) | MILP | Nodal Zonal DC | Thermal, RES |
| Nycander E, 2021 | Nordic System | Nordic region | Hour | ODIN | MILP | Multi-node DC | Thermal, Nuclear . Hydro, RES |
| Katsanevakis , 2022 | Cyprus | Europe | Hour | DISPA-SET High-RES simulation | MILP | Multi &Single-Node /microgrid | PV, Wind, diesel, storage |
| Unneweher, 2022 | ENTSO-E | Germany & EU countries | Hour | PyPSA-Eur | MILP | Full-network DC | Various generation technologies |
| Pickering B, 2022 | ENTSO-E | Europe | Hour | Full-system decarbonization | MILP | Multi Node | Various generation technologies |
| Van Ouwerkerk, 2022 | ENTSO-E | Germany | Hour | OS power system model comparison | MILP | Multi Node | German mix |
| Vieira DAG, 2022 | Real hydropower plant | Brazil | 30 minutes | Real-time UC | MILP | Hydro plant network | Hydro |
| Rateele TN, 2023 | Lesotho | Africa | Hour | ED | LP | Simplified grid | Thermal, Hydro, VRE |
| Jiang Y, 2024 | PJM/MISO | US | Hour | Market offer optimization | MILP | Nodal DC | Thermal, gas, RES |
| Kleanthis N, 2025 | ENTSO-E | Europe | Hour | Capacity expansion + PCM | MILP, PCM | Zonal and Multi-node DC | Thermal, RES, Storage |
| Silva WN, 2025 | Brazil’s electricity market | Brazil | 30 minutes | Market structure review | LP | Multi-Node hydro-thermal | Hydro, thermal |

To fully comprehend the application of modelling to the UC problem, the literature was evaluated according to the type of uncertainty modelling (deterministic, robust, stochastic, and chance-constrained UC). In the deterministic UC approach, all inputs are presumably known with certainty and represented by a single forecast value. In the case of robust UC, solutions that remain feasible within a predetermined uncertainty set are sought for. Robust techniques can be too conservative and raise operating expenses, even while they improve reliability. Stochastic UC minimizes expected values, i.e. predicted operating costs while reflecting trade-offs between risk and efficiency by modelling uncertainty through a variety of scenarios with corresponding probabilities (Wuijts, RH, 2024). Chance-constrained UC is a variation of stochastic modelling that provides an organized balance between risk and expense by limiting the likelihood of constraint breaches (such as load balance or reserve sufficiency). Deterministic modelling is widely used and is computationally efficient, particularly for day-ahead markets. Conventional assumptions or reserve requirements are frequently used in deterministic UC as a hedge against uncertainty, which can lead to suboptimal costs or reliability problems. On the other hand, stochastic and chance-constrained UC lead to more accurate solutions but have enhanced computational complexity. Literature has been organized with a categorization per i) modelling parameters (Objective Function, Constraints, Method used, Tools used, other variables, type of problem solved), ii) time frame resolution, and iii) Case studies. The analysis of recent publications with substantially available information, showed that i) Studies on central CSO are apparent in recent cases in the US, Brazil, as well as the EU (ENTSO-E, Nord Pool); ii) Energy system planning, market analysis, and policy-driven models are where real-system studies are most prevalent (EU, Germany, Poland, Brazil) and iii) Deterministic UC modelling is predominant on an hourly time frame as in (Silva WN, 2025), whereas stochastic and robust modelling is evolving to be more widespread in cases in Sweden (Goop J, 2021) US (Jiang Y, 2024) and China (Feng Y, 2024) and for profit maximization and decision uncertainty (Lotfi H, 2024).

In the future, SCUC modelling will need to meet more stringent environmental and energy policy objectives. This challenge can be addressed in a number of ways. Computational modelling can be improved, for example, by incorporating ISO-specific constraints or by incorporating novel techniques like AI and ML. The cooperation between regions could also support the alignment of individual markets. Furthermore, the SCUC process can be in accordance with decarbonization goals if power generation is co-optimized with hydrogen storage, EVs, and synthetic fuels (Latify MA, 2025), (Ahmed I, 2026).

4. DISCUSSION

A total of 150 peer-reviewed literature articles from a variety of 64 different journals/ sources, were studied. Widely used models and tools adjacent to UCED were identified. Power optimization methods and tools were categorized into (i) traditional, (ii) nature-inspired algorithms (NIAs), (iii) AI, (iv) Hybrid Artificial Intelligent Algorithms, and (v) Quantum Computing. Bibliometrics on the basic database outlined the evolution of relevant UC terms throughout recent years. A distinction between the pros and cons of UC prevailing solving methods (PSO, DE, GWO, JA, COA, FWA, CaB, GA, and CS) was made based on convergence speed, solution accuracy, computational complexity, and scalability. A literature review on UCED methods and tools focusing on “current years” (2015 – 2026) was made, exclusively on 62 peer-reviewed literature regarding worldwide regions. Literature has been filtered to deduce those with the most representative and ample data on application in IEEE systems as well as on realistic system applications, on UCED-related issues. Those were examined per ascending timescale and per modelling methodology, i.e., deterministic, robust, and stochastic.

UCED appeared challenged by subjects that need further discussion: i) Multiple case study evidence is necessary to reinforce the credibility of UCED or solely UC modelling; ii) The lack of OS data and licenses can be a drawback. Data availability and quality need enhancement. Iii) Affordable, reliable, and sustainable energy, i.e., the UN goal SDG7, could be enhanced by UCED optimization. In this view, power dispatch from DER appears vital; time constraints might be equally important as optimization accuracy, as in the case of emergencies, adequate storage facilities must be in place.

Regarding the UC solving, ramping limitations, reserve constraints, and the constraints of the transmission system should not be oversimplified, otherwise the UC will be qualitatively degraded. SCUC modelling will need to meet more stringent environmental and energy policy objectives in the future and solutions might be drawn through cross-regional cooperation as well as co-optimization with hydrogen storage or other options.

UCED models must adjust to market rules, regional factors, and security constraints while sustaining profit maximization. Societal impacts could be integrated and combined with environmental and energy policy incentives and risk readiness. Future energy systems require UCED-based insights by means of demand-side flexibility and sustainable integration of VRE. Models on UCED extend far beyond the technical scheduling of generators. They offer deep insights into the economic, environmental, and operational consequences of energy policies.

UC modelling at the country and regional level was examined. The problem of UC in centrally dispatched markets was underrepresented in the literature. However, those mostly studied engaged CSOs in the US.

5. Conclusion

Traditional approaches for solving the combined UCED problem (LP, MILP, MINLP) remain leading in real-system applications due to their robustness, transparency, and compatibility with commercial solvers such as Gurobi and CPLEX.

There is a stable increase in UCED related publications from 2015 to 2026, with a rise in relative OS publications since 2020. NIAs, including PSO, GWO, GA, JA, COA, and FWA, are widely applied in academic studies because of their flexibility and ability to handle non-convexities, despite limitations related to convergence speed and scalability. Until 2018, studies were primarily tackling operational topics such as OPF, SC/SCUC, short-term scheduling, and CEED. From 2019 onward, optimization tools addressed in review articles show a noteworthy shift towards addressing uncertainty management, flexibility, and energy storage. There is also a growing concern about VRE and the increasing importance of system adaptability. Research also increasingly assesses electricity markets and policy implications, alongside emerging technologies such as DR and EV. By 2026, literature highlights the use of advanced computational methods, including AI and QC, as well as broader multi-objective optimization approaches, indicating a shift from deterministic, classical UCED methods towards more holistic frameworks.

It is noteworthy that the parameters of flexibility, energy storage, and uncertainty management have developed throughout time to be common concerns for both CSOs and GENCO/market participants. Energy and environmental policy implications and uncertainty management have stood out as parameters of concern throughout all years in optimization methods and tools.

The application of UC in central dispatch markets and realistic system applications on UCED related issues, on an hourly timescale, is addressed in a limited case study number, yet more on a sub-hourly scale. Such studies are most common in energy system planning, market analysis, and policy-driven models (EU, Germany, Poland, Brazil). Deterministic UC modelling outweighs, whereas stochastic and robust modelling have begun appearing more commonly. Studies on central CSO are evident in recent cases in the US, Brazil, and also in EU regions (ENTSO-E, Nord Pool). Modelling on larger than hourly time scales on IEEE systems is mentioned extensively on IEEE bus 14, 30-, 40-, 118-, 140-unit systems and IEEE RTS. On a multi-year time horizon, real systems tested with long-term Energy System Optimization Models (ESOM) typically concern OSeMOSYS, POLES, PRIMES, and PLEXOS.

The UC optimization in centrally dispatched markets is being tackled in the literature by application of a plethora of models and tools, mostly in a deterministic environment and in some cases on an hourly or even sub-hourly basis. The existing modelling framework still needs to be advanced to transfer the application from IEE systems to real systems. Computational challenges can be faced by the combination and soft linking of models. Multiscale models (PLEXOS®, OSeMOSYS, FlexTool) and open-source approaches emerge as key enablers for country- and regional-level UC modelling in centrally dispatched markets.

COMPETING INTERESTS DISCLAIMER:

Authors have declared that they have no known competing financial interests OR non-financial interests OR personal relationships that could have appeared to influence the work reported in this paper.

Disclaimer (Artificial intelligence)

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

References

1.	Apon, H.J.; Ahmed, A.; Rahman Adib, A.U.; Ahshan, R. (2025) Integrated planning of emission reduction-based unit commitment and generation dispatch solution for sustainable power systems. Electr. Power Syst. Res. 2025, 244, 111535. ISSN 0378-7796, https://doi.org/10.1016/j.epsr.2025.111535

2.	Singh, A.; Khamparia, A.; Al-Turjman, F. (2024) A hybrid evolutionary approach for multi-objective unit commitment problem in power systems. Energy Rep. 2024, 11, Pages 2439-2449, https://doi.org/10.1016/j.egyr.2024.02.004

3.	Marzbani F, Abdelfatah A. (2024) Economic Dispatch Optimization Strategies and Problem Formulation: A Comprehensive Review. Energies 2024, 17(3), 550; https://doi.org/10.3390/en17030550

4.	Goforth T, Levin T, Nock D. (2025) Incorporating energy justice and equity objectives in power system models. Renew Sustain Energy Rev. 2025;210:115155. https://doi.org/10.1016/j.rser.2024.115155.

5.	Kleanthis N, Stavrakas V, Flamos A. (2025) Bidirectional soft-linking of a Capacity Expansion Model with a Production Cost Model to evaluate the feasibility of transition pathways towards carbon neutrality in the power sector. Appl Energy. 2025; (Vol. 378, Part B) :124843. ISSN 0306-2619, https://doi:10.1016/j.apenergy.2024.124843.

6.	Kaya A, Conejo AJ, Rebennack S. (2025) Fifty years of power systems optimization. Eur J Oper Res. 2025; https://doi:10.1016/j.ejor.2025.05.022

7.	Cho S, Li C, Grossmann IE. Recent advances and challenges in optimization models for expansion planning of power systems and reliability optimization.(2022) Comput Chem Eng. 2022;165:107924. https://doi: 10.1016/j.compchemeng.2022.107924.

8.	Marcovecchio MG, Novais AQ, Grossmann IE. Deterministic optimization of the thermal Unit Commitment problem: A Branch and Cut search. (2014) Comput Chem Eng. 2014;67:53-68. https://doi:10.1016/j.compchemeng.2014.03.009.

9.	Happ HH. Optimal power dispatch. IEEE Trans Power Appar Syst. 1974;PAS-93(3):820-830. https://doi:10.1109/TPAS.1974.293981.

10.	Happ HH. Optimal power dispatch: A comprehensive survey. IEEE Trans Power Appar Syst. 1977;96(3):841-854. https://doi:10.1109/T-PAS.1977.32397.

11.	Stott B, Marinho JL, Alsac O. (1979), Review of linear programming applied to power system rescheduling. In: IEEE Conference Proceedings Power Industry Computer Applications Conference, 1979. PICA-79. Cleveland, OH, USA;1979. P.142-154. https://doi:10.1109/PICA.1979.720058.

12.	Chowdhury BH, Rahman S. (1990) A review of recent advances in economic dispatch. IEEE Trans Power Syst. 1990;5(4):p.1248-1259. https://doi:10.1109/59.99376.

13.	Sterling MJH, 1984, Irving MR. Optimisation methods for economic dispatch in electric power systems. Trans Inst Meas Control. 1984;6(4):p.247-252. https://doi:10.1177/014233128400600409.

14.	Basu M. (2019) Economic environmental dispatch of solar-wind-hydro-thermal power system. Renew Energy Focus.2019;30:p.107-122. https://doi:10.1016/j.ref.2019.04.002.

15.	Motta VN, Anjos MF, Gendreau M. (2024) Survey of optimization models for power system operation and expansion planning with demand response. Eur J Oper Res. 2024;312(2): p.401-412. https://doi: 10.1016/j.ejor.2023.01.019.

16.	Manikandan TR, Thangavelu V. (2023) Combined economic and emission power dispatch control using substantial augmented transformative algorithm. Intell Autom Soft Comput. 2023;35(1): p.431-447. https://doi:10.32604/iasc.2023.026546.

17.	Asgharian V, Hajebrahimi A, Abdelaziz M, Kamwa I. (2022) Coordinated generation expansion planning & energy storage planning model of the IPP’s participation in the electricity markets. Electric Power Systems Research. 2022; 205:107743. https://doi:10.1016/j.epsr.2021.107743.

18.	Gómez S, Olmos L. (2024) Coordination of generation and transmission expansion planning in a liberalized electricity context — coordination schemes, risk management, and modelling strategies: A review. Sustainable Energy Technologies and Assessments. 2024; 64:103731. https://doi: 10.1016/j.seta.2024.103731.

19.	Jiang Y, Sioshansi R. (2024) A computationally efficient approach to optimizing offers in centrally committed electricity markets. Eur J Oper Res. 2024;317(1):25-42. https://doi:10.1016/j.ejor.2024.01.040.

20.	Ahlqvist V, Holmberg P, Tangerås T. (2022) A survey comparing centralized and decentralized electricity markets. Energy Strategy Rev. 2022;40:100812. https://doi:10.1016/j.esr.2022.100812.

21.	Silva WN, Rego EE, Vieira GGT, Lourenço LFN Salles MBC., (2025). An overview of Brazil’s electricity market: Planning, dispatch models, pricing, and modernization. Util Policy. 2025; 96:102007. https://doi:10.1016/j.jup.2025.102007.

22.	Ringkjøb HK, Haugan PM, Solbrekke IM. (2018). A review of modelling tools for energy and electricity systems with large shares of variable renewables. Renew Sustain Energy Rev. 2018;96:440-459. https://doi:10.1016/j.rser.2018.08.002.

23.	Fattahi A, Sijm J, Faaij A.A (2020) systemic approach to odelli integrated energy system odelling tools: A review of national models. Renew Sustain Energy Rev.2020;133:110195. https://doi:10.1016/j.rser.2020.110195.

24.	Song D, Meng W, Dong M, Yang J, Wang J, Chen X, et al. (2022) A critical survey of integrated energy system: Summaries, meth-odologies and analysis. Energy Convers Manag. 2022;266:115863. https://doi:10.1016/j.enconman.2022.115863.

25.	Haugen M, Blaisdell-Pijuan PL, Botterud A, Levin T, Zhou Z, Belsnes M, et al. (2024) Power market models for the clean energy transition: State of the art and future research needs. Appl Energy. 2024;357:122495. https://doi:10.1016/j.apenergy.2023.122495.

26.	Gupta R and Jain P, (2023). A review of Market Based Economic Dispatch in India for uniform electricity pricing. International Conference on Power, Instrumentation, Energy and Control (PIECON), Aligarh, India, 2023, pp. 1-6, Electronic ISBN:979-8-3503-9976-9, https://doi: 10.1109/PIECON56912.2023.10085854.

27.	Wuijts RH, van den Akker, M. & van den Broek, M. (2024) Effect of modelling choices in the unit commitment problem. Energy Syst 15, 1–63 (2024). Publisher: Springer Nature, https://doi.org/10.1007/s12667-023-00564-5

28. Latify MA, Mokhtari A., Alavi-Eshkaftaki A., Najafabadi FR, Hashemian SN, Khaleghizadeh A., et al (2025). Security-constrained unit commitment: Modelling, solutions and evaluations, Applied Energy, Volume 390, 2025, 125796, ISSN 0306-2619, https://doi.org/10.1016/j.apenergy.2025.125796.

29.	Victor N, Nichols C. (2024). Future of hydrogen in the U.S. energy sector: MARKAL odelling results. Appl Energy Combust Sci. 2024;18:100259. https://doi:10.1016/j.jaecs.2024.100259.

30.	Luxembourg SL, Salim SS, Smekens K, Dalla Longa F.& van der Zwaan B. (2025) TIMES-Europe: An integrated energy system model for odelling Europe’s energy and climate challenges. Environ Model Assess. Volume 30, pages 1–19, (2025). https://doi:10.1007/s10666-024-09976-8.

31.	Goop J, Nyholm E, Odenberger M, Johnsson F. (2021) Impact of electricity market feedback on investments in solar photovoltaic and battery systems in Swedish single-family dwellings. Renew Energy. 2021;163: p.1078-1091. https://doi:10.1016/j.renene.2020.06.153.

32.	Lund H, Thellufsen JZ, Østergaard PA, Sorknæs P, Skov IR, Mathiesen BV. (2021) EnergyPLAN – Advanced analysis of smart energy systems. Smart Energy. 2021;1:100007. https://doi:10.1016/j.segy.2021.100007.

33.	Wiese F, Bramstoft R, Koduvere H, Pizarro Alonso A, Balyk O, Kirkerud JG, et al. (2018) Balmorel open source energy system model. Energy Strat Rev 2018;20: p.26-34. https://doi: 10.1016/j.esr.2018.01.003.

34.	Handayani K, Overland I, Suryadi B, Vakulchuk R. (2023) Integrating 100% renewable energy into electricity systems: A net-zero analysis for Cambodia, Laos, and Myanmar. Energy Rep. 2023;10:p.4849-4869. https://doi:10.1016/j.egyr.2023.11.005.

35.	Nicoli M, Gracceva F, Lerede D, Savoldi L. (2022) Can We Rely on Open-Source Energy System Optimization Models? Energies 2022, 15(18), 6505; https://doi.org/10.3390/en15186505

36.	Hamdi M, El Salmawy HA, Ragab R. (2024) Incorporating operational constraints into long-term energy planning: The case of the Egyptian power system under high share of renewables. Energy. 2024;300:131619. https://doi:10.1016/j.energy.2024.131619.

37.	Després J, Mima S, Kitous A, Criqui P, Hadjsaid N, Noirot I. (2016) Storage as a flexibility option in power systems with high shares of variable renewable energy sources: a POLES-based analysis. Energy Econ. 2017;64:638-650. https://doi:10.1016/j.eneco.2016.03.006.

38.	Capros P, Kannavou M, Evangelopoulou S, Petropoulos A, Siskos P, Tasios N, Zazias G, DeVita A. (2018) Outlook of the EU energy system up to 2050: The case of scenarios prepared for European Commission’s “clean energy for all Europeans” package using the PRIMES model. Energy Strat Rev. 2018;22: p.255-263. https://doi:10.1016/j.esr.2018.06.009.

39.	Nycander E, Söder L. (2022) An open dispatch model for the Nordic power system. Energy Strat Rev. 2022;39:100775. https://doi:10.1016/j.esr.2021.100775.

40.	Chowdhury AFMK, Kern J., Dang TD., Galelli S. (2020). PowNet: A Network-Constrained Unit Commitment/Economic Dispatch Model for Large-Scale Power Systems Analysis. J Open Res Softw. 2020;8:5. https://doi:10.5334/jors.302.

41.	Zeljko M, Aunedi M, Slipac G, Jakšić D. (2020) Applications of Wien Automatic System Planning (WASP) Model to Non-Standard Power System Expansion Problems. Energies. 2020;13:1392. https://doi:10.3390/en13061392.

42.	Katsanevakis A, Konstantinidis D, Karagiannis G, Ganias A., Karagiorgis G., Partasides G. et al (2022) Simulation of an isolated system odelling at high RES penetration coupled with storage. Discov Energy. 2022;2:1. https:// doi:10.1007/s43937-022-00006-w

43.	Gholami K., Arif MT, Haque ME, Arefi A., Muyeen S.M,(2025) Comprehensive review of cutting-edge virtual power plant advancements for flexibility enhancement in future power grids, Energy Strategy Reviews, Volume 62, 2025, 101959, ISSN 2211-467X, https://doi.org/10.1016/j.esr.2025.101959

44.	Majanne Y, Björkqvist T, Vilkko M. Scenarios for future power system development in Finland. IFAC-PapersOnLine. 2022;55(9):6-11. https://doi:10.1016/j.ifacol.2022.07.002.

45.	Unnewehr JF, Schäfer M, Weidlich A. (2022) The value of network resolution – A validation study of the European energy system model PyPSA-Eur. In: 2022 Open Source Modelling and Simulation of Energy Systems (OSMSES), Aachen; 2022. P1-7. https://doi:10.1109/OSMSES54027.2022.9769123.

46.	Majidi H, Hayati MM, Breyer C, Mohammadi-ivatloo B, Honkapuro S, Karjunen H et al (2025), Overview of energy modeling requirements and tools for future smart energy systems, Renew Sustain Energy Rev. 2025;12:115367,ISSN 1364-0321, https://doi.org/10.1016/j.rser.2025.115367

47.	Niet T, Shivakumar A, Gardumi F, Usher W, Williams E, Howells M. (2021) Developing a community of practice around an open source energy modelling tool. Energy Strategy Reviews 2021;35:100650. https://doi:10.1016/j.esr.2021.100650.

48.	Hart WE, Laird C, Watson JP, Woodruff DL. (2012) Introduction. In: Pyomo – Optimization Modelling in Python. Springer Optimization and Its Applications, vol 67. Boston, MA: Springer; 2012. https://doi:10.1007/978-1-4614-3226-5_1.

49.	Pickering B, Lombardi F, Pfenninger S. (2022) Diversity of options to eliminate fossil fuels and reach carbon neutrality across the entire European energy system. Joule. 2022;6(6):p:1253-1276. https://doi:10.1016/j.joule.2022.05.009.

50.	Gandhi O, Rodríguez Gallegos C, Srinivasan D. (2016) Review of optimization of power dispatch in renewable energy system. In: 2016 IEEE Innovative Smart Grid Technologies – Asia (ISGT-Asia); 2016. P. 250-257. Corpus ID: 28287742, https://doi:10.1109/ISGT-Asia.2016.7796394.

51.	Ahmed I, Rehan M., Alqahtani M., Khalid M., (2026). A systematic review on AI-driven computational strategies for sustainable power systems, Energy Strategy Reviews, Volume 63, 2026, 102041, ISSN 2211-467X, https://doi.org/10.1016/j.esr.2026.102041

52.	Sulaiman MH, Mustaffa Z, Saari MM, Daniyal H. (2020) Barnacles Mating Optimizer: A new bio-inspired algorithm for solving engineering optimization problems, Engineering Applications of Artificial Intelligence, 2020;87:103330, ISSN 0952-1976, https://doi.org/10.1016/j.engappai.2019.103330.

53.	Pandit N, Tripathi A, Tapaswi S, Pandit M. (2012) An improved bacterial foraging algorithm for combined static/dynamic environmental economic dispatch. Appl Soft Comput. 2012;12(11): p.:3500-3513. https://doi:10.1016/j.asoc.2012.06.011.

54.	Shaheen AM, Hasanien HM, Alkuhayli A. (2021) A novel hybrid GWO-PSO optimization technique for optimal reactive power dispatch problem solution. Ain Shams Eng J. 2021;12(1): p.621-630. https://doi:10.1016/j.asej.2020.07.011.

55.	Basak S, Bhattacharyya B, Dey B. (2022). Dynamic economic dispatch using hybrid CSAJAYA algorithm considering ramp rates and diverse wind profiles, Intelligent Systems with Applications, November 2022;16:200116, ISSN 2667-3053, https://doi:10.1016/j.iswa.2022.200116.

56.	Asif M, Amin A., Jamil U, Ahsan M, Bafakeeh O, Mahmood A. Ahmed U. Razzaq S, Mahdi FP (2024). Combined emission economic dispatch using quantum-inspired particle swarm optimization and its variants. Energy Exploration & Exploitation 2024, 42(5):1602-1644. https://doi:10.1177/01445987241235419.

57. Yuan G, Yang W. Study on optimization of economic dispatching of electric power system based on Hybrid Intelligent Algorithms (PSO and AFSA). Energy. 2019;183: p:926-935. ISSN 0360-5442, https://doi:10.1016/j.energy.2019.07.008.

58.	Morstyn T, Wang X. (2024) Opportunities for quantum computing within net-zero power system optimization. Joule. 2024;8(6): p: 1619-1640. https://doi:10.1016/j.joule.2024.03.020.

59.	Weibezahn J, Kendziorski M. (2019) Illustrating the Benefits of Openness: A Large-Scale Spatial Economic Dispatch Model Using the Julia Language. Energies. 2019;12:1153. https://doi:10.3390/en12061153.

60.	Groissböck M. (2019) Are open source energy system optimization tools mature enough for serious use? Renew Sustain Energy Rev. 2019;102: p234-248. https://doi:10.1016/j.rser.2018.11.020.

61.	Pfenninger S, Hirth L, Schlecht I, et al. (2018) Opening the black box of energy modelling: Strategies and lessons learned. Energy Strat Rev. 2018;19: p63-71. https://doi:10.1016/j.esr.2017.12.002.

62.	Van Ouwerkerk J, Hainsch K, Candas S, Muschner C., Buchholz S., Günther S. et al. (2022) Comparing open source power system models – A case study focusing on fundamental modelling parameters for the German energy transition. Renew Sustain Energy Rev. 2022;161:112331. https://doi:10.1016/j.rser.2022.112331.

63.	Schöpfel J, Farace DJ, Prost H, Zane A. (2019) Data Papers as a New Form of Knowledge Organization in the Field of Research Data.Comput Sci. 2019. https://doi.10.5771/0943-7444-2019-8-622.

64.	Guerra K, Haro P, Gutiérrez RE, Gómez-Barea A. Data for the modelling of the future power system with a high share of variable renewable energy (2022) Data Brief. 2022;42:108095. https://doi:10.1016/j.dib.2022.108095.

65.	Guerra K, Haro P, Gutiérrez RE, Gómez-Barea A. (2022) Facing the high share of variable renewable energy in the power system: Flexibility and stability requirements. Appl Energy. 2022;310:118561. https://doi:10.1016/j.apenergy.2022.118561.

66.	Ren B, Jiang C. (2009) A review on the economic dispatch and risk management considering wind power in the power market. Renew Sustain Energy Rev. 2009;13(8):2169-2174. https://doi:10.1016/j.rser.2009.01.013.

67.	Foley AM, Leahy PG, Marvuglia A, McKeogh EJ. (2012) Current methods and advances in forecasting of wind power generation. Renew Energy. 2012;37(1):1-8. https://doi:10.1016/j.renene.2011.05.033.

68.	Inman RH, Pedro HTC, Coimbra CFM. (2013) Solar forecasting methods for renewable energy integration. Prog Energy Combust Sci. 2013;39(6):535-576. https://doi:10.1016/j.pecs.2013.06.002.

69.	Swarup KS, Yamashiro S. (2003) A genetic algorithm approach to generator unit commitment. Int J Electr Power Energy Syst. 2003;25(9):679-687. https://doi:10.1016/S0142-0615(03)00003-6.

70.	Mahor A, Prasad V, Rangnekar S. (2009) Economic dispatch using particle swarm optimization: A review. Renew Sustain Energy Rev. 2009;13(8):2134-2141. https://doi:10.1016/j.rser.2009.03.007.

71.	Peng M, Liu L, Jiang C. (2012) A review on the economic dispatch and risk management of the large-scale plug-in electric vehicles (PHEVs)-penetrated power systems. Renew Sustain Energy Rev. 2012;16(3):p1508-1515. https://doi:10.1016/j.rser.2011.12.009.

72.	Amudha A, Christober Asir Rajan C.(2013) Generator scheduling under competitive environment using Memory Management Algorithm, Alexandria Engineering Journal, V52(3) 2013, p.337-346, ISSN 1110-0168, https://doi.org/10.1016/j.aej.2013.05.004.

73.	Nappu M, Arief A, Bansal RC. (2014) Transmission management for congested power system: A review of concepts, technical challenges and development of a new methodology. Renew Sustain Energy Rev. 2014;38:p572-580. https://doi:10.1016/j.rser.2014.05.089.

74.	Simoglou CK, Kardakos EG, Bakirtzis EA, Chatzigiannis D.I., Vagropoulos S.I., Ntomaris A.V., Biskas P. (2014). An advanced model for the efficient and reliable short-term operation of insular electricity networks with high renewable energy sources penetration. Renew Sustain Energy Rev. 2014;38:p415-427. https://doi:10.1016/j.rser.2014.06.015.

75.	Norouzi MR, Ahmadi A, Esmaeel Nezhad A, Ghaedi (2014) A. Mixed integer programming of multi-objective security-constrained hydro/thermal unit commitment. Renew Sustain Energy Rev. 2014;29:p911-923. https://doi:10.1016/j.rser.2013.09.020.

76.	Karami M, Shayanfar HA, Aghaei J, Ahmadi A. (2013) Scenario-based security-constrained hydrothermal coordination with volatile wind power generation. Renew Sustain Energy Rev. 2013;28:p726-737. https://doi:10.1016/j.rser.2013.07.052.

77.	Raza MQ, Khosravi A. (2015) A review on artificial intelligence-based load demand forecasting techniques for smart grid and buildings. Renew Sustain Energy Rev. 2015;50:p1352-1372. https://doi:10.1016/j.rser.2015.04.065.

78.	Antonanzas J, Osorio N, Escobar R, Urraca R, Martinez-de-Pison FJ, Antonanzas-Torres F. (2016) Review of photovoltaic power forecasting. Solar Energy. 2016;136:78-111. https://doi:10.1016/j.solener.2016.06.069.

79.	Tan KM, Ramachandaramurthy VK, Yong JY. (2016) Integration of electric vehicles in smart grid: A review on vehicle-to-grid technologies and optimization techniques. Renew Sustain Energy Rev. 2016;53:p720-732. https://doi:10.1016/j.rser.2015.09.012.

80.	Capitanescu F. (2016) Critical review of recent advances and further developments needed in AC optimal power flow. Electr Power Syst Res. 2016;136:57-68. https://doi:10.1016/j.epsr.2016.02.008.

81.	Abujarad SY, Mustafa MW, Jamian JJ. (2017) Recent approaches of unit commitment in the presence of intermittent renewable energy resources: A review. Renew Sustain Energy Rev. 2017;70:p215-223. https://doi:10.1016/j.rser.2016.11.246.

82.	Nazari-Heris M. A comprehensive review of heuristic optimization algorithms for optimal combined heat and power dispatch from economic and environmental perspectives. (2018) Renew Sustain Energy Rev. 2018;81(2):2128-2143. https://doi:10.1016/j.rser.2017.06.024.

83.	Koltsaklis NE, Dagoumas AS. State-of-the-art generation expansion planning: A review. (2018) Appl Energy. 2018;230:p563-589. https://doi:10.1016/j.apenergy.2018.08.087.

84.	Ahmed A, Khalid M. A review on the selected applications of forecasting models in renewable power systems. (2019) Renew Sustain Energy Rev. 2019;100:9-21. https://doi:10.1016/j.rser.2018.09.046.

85.	Kong J, Skjelbred HI, Fosso OB. (2020) An overview on formulations and optimization methods for the unit-based short-term hydro scheduling problem. Electr Power Syst Res. 2020;178:106027. https://doi:10.1016/j.epsr.2019.106027.

86.	Ahmad T, Zhang H, Yan B. (2020) A review on renewable energy and electricity requirement forecasting models for smart grid and buildings. Sustain Cities Soc. 2020;55:102052. https://doi:10.1016/j.scs.2020.102052.

87.	Bagherian MA, Mehranzamir K. (2020) A comprehensive review on renewable energy integration for combined heat and power production. Energy Convers Manag. 2020;224:113454. https://doi:10.1016/j.enconman.2020.113454.

88.	Qiu H, Gu W, Liu P, Sun Q, Wu Z, Lu X. (2022) Application of two-stage robust optimization theory in power system scheduling under uncertainties: A review and perspective. Energy. 2022;251:123942. https://doi:10.1016/j.energy.2022.123942.

89.	Thirunavukkarasu GS, Seyedmahmoudian M, Jamei E, Horan B, Mekhilef S, Stojcevski A. (2022) Role of optimization techniques in microgrid energy management systems—A review. Energy Strategy Rev. 2022;43:100899. https://doi:10.1016/j.esr.2022.100899.

90.	Kunya AB, Abubakar AS, Yusuf SS. (2023) Review of economic dispatch in multi-area power system: State-of-the-art and future prospective. Electr Power Syst Res. 2023;217:109089. https://doi:10.1016/j.epsr.2022.109089.

91.	Golestan S, Habibi MR, Mousazadeh Mousavi SY, Guerrero JM, Vasquez JC. (2023) Quantum computation in power systems: An overview of recent advances. Energy Rep. 2023;9:584-596. https://doi:10.1016/j.egyr.2022.11.185.

92.	Zhang J, Liu Y, Tian D, Yan J. (2015) Optimal power dispatch in wind farm based on reduced blade damage and generator losses. Renew Sustain Energy Rev. 2015;44:64-77. https://doi:10.1016/j.rser.2014.12.008.

93.	Moghaddas Tafreshi SM, Ranjbarzadeh H, Jafari M, Khayyam H. (2016) A probabilistic unit commitment model for optimal operation of plug-in electric vehicles in microgrid. Renew Sustain Energy Rev. 2016;66:p934-947. https://doi:10.1016/j.rser.2016.08.013.

94.	Taylor JA, Dhople SV, Callaway DS (2016). Power systems without fuel. Renew Sustain Energy Rev. 2016;57:p.1322-1336. https://doi: 10.1016/j.rser.2015.12.083

95.	Nazari-Herris M, Mohammadi-Ivatloo B, Gharehpetian GB. (2017) Short-term scheduling of hydro-based power plants considering application of heuristic algorithms: A comprehensive review. Renew Sustain Energy Rev. 2017;74:p116-129 https:// doi:10.1016/j.rser.2017.02.043.

96.	Wang Y, Wang S, Wu L. (2017) Distributed optimization approaches for emerging power systems operation: A review. Electr Power Syst Res. 2017;144:127-135. https://doi:10.1016/j.epsr.2016.11.025.

97.	Lu X, Zhou K, Zhang X, Yang S. (2018) A systematic review of supply and demand side optimal load scheduling in a smart grid environment. J Clean Prod. 2018;203:p757-768. https://doi:10.1016/j.jclepro.2018.08.301.

98.	Cornelius A, Bandyopadhyay R, Patiño-Echeverri D. (2018) Assessing environmental, economic, and reliability impacts of flexible ramp products in MISO’s electricity market. Renew Sustain Energy Rev. 2018;81(2):p2291-2298. https://doi:10.1016/j.rser.2017.06.037.

99.	Mahdi FP, Vasant P, Kallimani V, Watada J, Yeoh SFW, Abdullah-Al-Wadud M. (2018) A holistic review on optimization strategies for combined economic emission dispatch problem. Renew Sustain Energy Rev. 2018;81(2):p3006-3020. https://doi:10.1016/j.rser.2017.06.111.

100.	Håberg M. (2019) Fundamentals and recent developments in stochastic unit commitment. Int J Electr Power Energy Syst. 2019;109:38-48. https://doi:10.1016/j.ijepes.2019.01.037.

101.	Chimnoy L, Iniyan S, Goic R. (2019) Modelling wind power investments, policies and social benefits for deregulated electricity market – A review. Appl Energy. 2019;242:p364-377. https://doi:10.1016/j.apenergy.2019.03.088.

102.	Scuzziato MR, Finardi EC, Frangioni A. (2021) Solving stochastic hydrothermal unit commitment with a new primal recovery technique based on Lagrangian solutions. Int J Electr Power Energy Syst. 2021;127:106661. https://doi:10.1016/j.ijepes.2020.106661.

103.	Abdi H. (2021) Profit-based unit commitment problem: A review of models, methods, challenges, and future directions. Renew Sustain Energy Rev. 2021;138:110504. https://doi:10.1016/j.rser.2020.110504.

104.	Maneesha A, Swarup KS (2021). A survey on applications of Alternating Direction Method of Multipliers in smart power grids. Renew Sustain Energy Rev. 2021;152:111687. https://doi:10.1016/j.rser.2021.111687.

105.	Iychettira KK. (2021). Lessons for renewable integration in developing countries: The importance of cost recovery and distributional justice. Energy Res Soc Sci. 2021;77:102069. https://doi:10.1016/j.erss.2021.102069.

106.	Liu J, Ou M, Sun X, Chen J, Mi C, Bo R. (2022) Implication of production tax credit on economic dispatch for electricity merchants with storage and wind farms. Appl Energy. 2022;308:118318. https://doi:10.1016/j.apenergy.2021.118318.

107.	Vieira DAG, Costa EE, Campos PHF, Mendonça MO, Silva GRL.(2022)  A real-time nonlinear method for a single hydropower plant unit commitment based on analytical results of dual decomposition optimization. Renew Energy. 2022; 192:p513-525. https://doi.10.1016/j.renene.2022.04.080.

108.	Skolfield JK, Escobedo AR. (2021) Operations research in optimal power flow: A guide to recent and emerging methodologies and applications. Eur J Oper Res. 2022;300(2):387-404. https://doi:10.1016/j.ejor.2021.10.003.

109.	Psarros GN, Papathanassiou SA. (2023) Generation scheduling in island systems with variable renewable energy sources: A literature review. Renew Energy. 2023;205:p1105-1124. https://doi:10.1016/j.renene.2023.01.099.

110.	Aharwar A, Naresh R, Sharma V, Kumar V. (2023) Unit commitment problem for transmission system, models and approaches: A review. Electr Power Syst Res. 2023;223:109671. https://doi:10.1016/j.epsr.2023.109671.

111.	Liu H, Tang W. (2023) Quantum computing for power systems: Tutorial, review, challenges, and prospects. Electr Power Syst Res. 2023;223:109530. https://doi:10.1016/j.epsr.2023.109530.

112.	Eren Y, Küçükdemiral İ. (2024) A comprehensive review on deep learning approaches for short-term load forecasting. Renew Sustain Energy Rev. 2024;189(B):114031 .https://doi:10.1016/j.rser.2023.114031

113.	Sharifian Y, Abdi H. (2024) Multi-area economic dispatch problem: Methods, uncertainties, and future directions. Renew Sustain Energy Rev. 2024;191:114093. https://doi:10.1016/j.rser.2023.114093.

114.	Dahane AS, Sharma RB. (2024) Hybrid AC-DC microgrid coordinated control strategies: A systematic review and future prospect. Renew Energy Focus.2024;49:100553. https://doi:10.1016/j.ref.2024.100553.

115.	Hassan MH, Kamel S, Jurado F, Desideri U. (2024) Global optimization of economic load dispatch in large scale power systems using an enhanced social network search algorithm. Int J Electr Power Energy Syst. 2024;156:109719. https://doi:10.1016/j.ijepes.2023.109719.

116.	Olasoji AO, Oyedokun DTO., Omogoye SO, Thron C., (2024) Review of frequency response strategies in renewable-dominated power system grids: Market adaptations and unit commitment formulation, Scientific African, 2024;26,e02357,ISSN 2468-2276, https://doi.org/10.1016/j.sciaf.2024.e02357.

117.	Chicco G, Mazza A. (2020) Metaheuristic optimization of power and energy systems: underlying principles and main issues of the ‘rush to heuristics’. Energies. 2020;13(19):5097. https://doi: 10.3390/en13195097.

118.	Mandal PK. (2023) A review of classical methods and Nature-Inspired Algorithms (NIAs) for optimization problems. Results in Control and Optimization. 2023;13:100315. https://doi:10.1016/j.rico.2023.100315.

119.	Zheng, H., Jian, J., Yang, L., & Quan, R. (2016). A deterministic method for the unit commitment problem in power systems. Computers & Operations Research., February 2016, 66, p241-247,ISSN 0305-0548, https://doi:10.1016/j.cor.2015.01.012.

120.	Saravanan B, Kumar C, Kothari DP. (2016). A solution to unit commitment problem using fire works algorithm. International Journal of Electrical Power & Energy Systems, 2016;77:p.221-227. ISSN 0142-0615, https://doi:10.1016/j.ijepes.2015.11.030.

121.	Ali ES, Abd Elazim SM, Balobaid AS (2023). Implementation of coyote optimization algorithm for solving unit commitment problem in power systems. Energy. 2023;263(Pt A):125697. https://doi:10.1016/j.energy.2022.125697.

122.	Chaudhary V, Dubey HM, Pandit M, Salkuti SR. (2023) A chaotic Jaya algorithm for environmental economic dispatch incorporating wind and solar power. AIMS Energy. 2023;12(1):p1-30. https://doi:10.3934/energy.2024001.

123.	Pandit M, Dubey HM, Bansal JC. (2020) Editors, Nature Inspired Optimization for Electrical Power System, first ed., Springer: Singapore; 2020.https://doi:10.1007/978-981-15-4004-2

124.	Valencia-Rivera GH, Benavides-Robles MT. (2024) Vela Morales A, Amaya I, Cruz-Duarte JM, Ortiz-Bayliss JC, Avina-Cervantes JG. (2024) A systematic review of metaheuristic algorithms in electric power systems optimization. Appl Soft Comput. 2024;150:111047.  https://doi:10.1016/j.asoc.2023.111047.

125.	Montero L, Bello A, Reneses J. (2022) A review on the unit commitment problem: approaches, techniques, and resolution methods. Energies. 2022;15(4):1296. https://doi:10.3390/en15041296.

126.	Abu Zitar R, Al-Betar MA, Awadallah MA, Abu Doush I, Assaleh K. (2022) An intensive and comprehensive overview of JAYA algorithm, its versions and applications. Archives of Computational Methods in Engineering. 2022;29:763–792. https://doi:10.1007/s11831-021-09585-8.

127.	Venkatasatish R, Chittathuru D. (2023) Coyote optimization algorithm-based energy management strategy for fuel cell hybrid power systems. Sustainability. 2023;15(12):9638. https://doi:10.3390/su15129638.

128.	Li X-G, Han S-F, Gong C-Q. (2017) Analysis and improvement of fireworks algorithm. Algorithms. 2017;10(1):26. https://doi: 10.3390/a10010026.

129.	Castelli AF, Harjunkoski I, Poland J, Giuntoli M, Martelli E, Grossmann IE. (2024) Solving the security constrained unit commitment problem: Three novel approaches. International Journal of Electrical Power & Energy Systems. 2024;162:110213. https://doi:10.1016/j.ijepes.2024.110213.

130.	Sheng W, Li R, Yan T, Tseng ML, Lou J, Li L. (2023) A hybrid dynamic economics emissions dispatch model: Distributed renewable power systems based on improved COOT optimization algorithm. Renew Energy. 2023;204:493-506. https://doi:10.1016/j.renene.2023.01.010

131.	Zeng L, Xu J, Wang Y, Liu Y, Tang J, Wen M, Chen Z. (2023) Day-ahead interval scheduling strategy of power systems based on improved adaptive diffusion kernel density estimation. Int J Electr Power Energy Syst. 2023;147:108850. https://doi:10.1016/j.ijepes.2022.108850.

132.	Feng Y, Wei W, Tian Y, Mei S. (2024) Integrating day-ahead unit commitment and real-time dispatch for a bulk renewable-thermal-storage generation base. Journal of Energy Storage, 2024;93:112074, ISSN 2352-152X, https://doi.org/10.1016/j.est.2024.112074.

133.	Samarasinghe SL, Moghimi M, Kaparaju P. (2025) A review of modelling tools for net-zero emission energy systems, based on model capabilities, modelling criteria and model availability. Renew Energy Focus. 2025;53:100659. https://doi:10.1016/j.ref.2024.100659.

134.	Rateele TN, Thamae LZ. (2023) An optimization approach for the economic dispatch incorporating renewable energy resources into Lesotho power sources portfolio, Heliyon, V9(4), 2023, e14748, ISSN 2405-8440, https://doi:10.1016/j.heliyon.2023.e14748.

135.	Dimitriadis CN, Tsimopoulos EG, Georgiadis MC. (2023) Optimization-based economic analysis of energy storage technologies in a coupled electricity and natural gas market. J Energy Storage. 2023;58:106332. https://doi:10.1016/j.est.2022.106332.

136.	Olivos C, Valenzuela J. I(2023) Integrating Battery Energy Storage Systems in the UC Problem: A Review. Curr Sustain/Renew Energy Rep. 2023;10:45–57.https://doi:10.1007/s40518-023-00210-9.

137.	Tahir H. (2024) Optimization of energy storage systems for integration of renewable energy sources — A bibliometric analysis. J Energy Storage. 2024;94:112497. https://doi:10.1016/j.est.2024.112497.

138.	Hammerschmitt BK, Capeletti MB, Lucchese FC, Guarda FKG, Abaide ADR. (2024) Multi-objective optimization of thermal dispatch scheduling considering different complementary load levels. Sustainable Energy Technol Assess. 2024;69:103909. https://doi:10.1016/j.seta.2024.103909.

139.	Liu X, Conejo AJ, Constante Flores GE. (2023) Stochastic Unit Commitment: Model Reduction via Learning. Curr Sustainable Renewable Energy Rep 10, 36–44 (2023). https://doi.org/10.1007/s40518-023-00209-2

140.	Li P, Su J, Bai X. (2024) An objective feasibility pump method for optimal power flow with unit commitment variables. Electric Power Syst Res. 2024;236:110928. https://doi:10.1016/j.epsr.2024.110928.

141.	Chen K, Shao Q, Zhou B, Yang Q, Pan H, Jin T, Liu S. (2023) Optimization of multi-temporal generation scheduling in power system under elevated renewable penetrations: A review. Front Energy Res. 2023;10. https://doi:10.3389/fenrg.2022.1054597.

142.	Abdi, H. (2023). A Survey of Combined Heat and Power-Based Unit Commitment Problem: Optimization Algorithms, Case Studies, Challenges, and Future Directions. Mathematics, 11(19), 4170. https://doi.org/10.3390/math11194170.

143.	Lejeune MA, Dehghanian P, Ma W. (2024) Profit-based unit commitment models with price-responsive decision-dependent uncertainty. Eur J Oper Res. 2024;314(3):p1052-1064. https://doi:10.1016/j.ejor.2023.12.006.

144.	Lotfi H, Nikkhah  M.H., (2024) Multi-Objective Profit-Based Unit Commitment with Renewable Energy and Energy Storage Units Using a Modified Optimization Method, Sustainability 2024, 16(4), 1708; https://doi:10.3390/su16041708

145.	Plazas-Niño F, Tan N, Howells M, Foster V, Quirós-Tortós J. (2025) Uncovering the applications, developments, and future research directions of the open-source energy modelling system (OSeMOSYS): A systematic literature review, Energy for Sustainable Development, April 2025, Volume 85, id.101629; ISSN 0973-0826, Bibcode: 2025EsusD..8501629P https://doi.org/10.1016/j.esd.2024.101629.

146.	Anand, R., Aggarwal, D., & Kumar, V. (2017). A comparative analysis of optimization solvers. Journal of Statistics and Management Systems, 20(4), p.623–635. https://doi.org/10.1080/09720510.2017.1395182.

147.	Cebulla F, Fichter T. (2017) Merit order or unit-commitment: How does thermal power plant modelling affect storage demand in energy system models? Renew Energy. 2017;105:117-132. https://doi:10.1016/j.renene.2016.12.043

148.	Pluta M, Wyrwa A, Suwała W, Zyśk J, Raczyński M, Tokarski S. (2020) A generalized unit commitment and economic dispatch approach for analysing the Polish power system under high renewable penetration. Energies. 2020;13(8):1952. https://doi:10.3390/en13081952

149.	Shen X, Zhu X, Yuan Y, Luo Z, Zhang X, Liu Y. (2025) Optimal dispatch of power grids considering carbon trading and green certificate trading. Technologies. 2025;13(7):294. https://doi:10.3390/technologies13070294

150.	Krishnamurthy S, Adewuyi OB, Salimon SA.(2026)  Recent advances in artificial intelligence-based optimization for power system applications: A review of techniques, challenges, and future directions. Renewable and Sustainable Energy Reviews, 2026;(226,Part C): 116340, ISSN 1364-0321, https://doi.org/10.1016/j.rser.2025.116340.

Definitions, Acronyms, Abbreviations

| Notations/Symbols | Definition |
| --- | --- |
| CoalGen | Coal generation power unit |
| GasGen | Gas generation power unit |
| [number] – bus system | Approximation of an electric power system with [#number] of substations |
| Node | Points of aggregated production and/or consumption of energy |
| Cg | Costs related to the generator |
| ct | Costs related to penalties |
| ugt | Represents the unit status of the generator g at an interval t |
| Pgt | The power output of the generator |

| Acronym | Definition |
| --- | --- |
| ACOPF | Alternating Current Optimal Power Flow |
| AFSA | Artificial Fish Swarm Algorithm |
| AGC | Automatic Generation Control |
| AI | Artificial intelligence |
| AML | Algebraic Modelling Languages |
| ANN | Artificial Neural Networks |
| AS | Ancillary Services |
| ASFA | Artificial Fish Swarm Algorithm |
| BALMOREL | Partial equilibrium model |
| BESS | Battery Energy Storage Systems |
| BFO | Bacteria Foraging Optimization |
| BWO | Black Widow Optimisation |
| CO2 | Carbon Dioxide |
| CaB | Cut and Branch method |
| Calliope | Multiscale energy systems modelling |
| CEED | Combined Economic Emission Dispatch |
| CHP | Combined heat and power |
| COA | Coyote Optimization algorithm |
| CS | Cuckoo Search |
| CSO | Central System Operator |
| DC | Direct Current |
| DE | Differential Evolution |
| DER | Distributed Energy Resources |
| Dispa SET | Unit Commitment / Power Dispatch Model |
| DR | Demand Response |
| EAs | Evolutionary Algorithms |
| ECL | European Climate Law |
| ED | Economic Dispatch |
| EFOM | Energy Flow Optimization Model |
| ELIN | Electricity Investment Model |
| ELMOD | Deterministic linear/mixed-integer Dispatch model |
| EM | Electricity Market |
| EnergyPLAN | Advanced Analysis of Smart Energy Systems |
| ENTSO – E | European Network of Transmission System Operators for Electricity |
| EPEX-SPOT | EU Power Exchange |
| EPOD | European Power Dispatch Model |
| ESEM | EU Single Electricity Market |
| ESOM | Energy System Optimization Model |
| ESS | Energy Storage System |
| EU | European Union |
| EVs | Electric Vehicles |
| FAIRE | Findability, Accessibility, Interoperability, and Reusability |
| Flex Tool | Least-cost optimization tool |
| FWA | Fire Works Algorithm |
| GA | Genetic Algorithm |
| GAMS | General Algebraic Modelling System |
| GENCO | Generation Company |
| GenAI | Generative artificial intelligence |
| GEP | Generation Expansion Planning |
| GHG | Greenhouse Gas Emissions |
| GWO | Grey Wolf Optimization |
| IEA | International Energy Agency |
| IEA – ETSAP | Energy Technology Systems Analysis Program of IEA |
| IEEE | Institute of Electrical and Electronics Engineers |
| IEEE bus system | Approximation of an electric power system |
| IES | Integrated energy systems |
| IO | Interval optimization |
| IPTO | Independent Transmission System Operator |
| IRENA | International Renewable Energy Agency |
| ISO | Independent System Operator |
| JA | Jaya Algorithm |
| LP | Linear programming |
| MAELD | Economic dispatch in multi-area power system |
| MARKAL | Market Allocation Model |
| MATPOWER | Programming package |
| MBED | Market Based Economic Dispatch |
| MEDUSA | Model of Economic Dispatch and Unit Commitment for System Analysis |
| MHs | Metaheuristics |
| MILP | Mixed Integer Linear Program |
| MINLP | Mixed-integer nonlinear programming |
| MIQP | Mixed Integer Quadratic Programming |
| MIP | Mixed Integer Programming |
| ML | Machine Learning |
| MMA | Memory Management Algorithm |
| MOEED | Multi-Objective Economic Emission Dispatch |
| MS | Member State |
| MT | Modelling Type |
| MTU | Market Time Unit |
| NEMO | Next Energy Modelling System for Optimization |
| NIAs | Nature-Inspired Algorithms |
| NLP | Nonlinear programming |
| ODIN | Open Dispatch Model for the Nordic Power System |
| O&M | Operation and maintenance |
| OPF | Optimal power flow |
| OS | Open Source |
| OSeMOSYS | Global energy systems optimization model |
| PBUC | (Price or ) Profit-based Unit Commitment |
| PCM | Production Cost Modelling |
| PE | Power Capacity Expansion |
| PLEXOS | Integrated Energy Model |
| POLES | Prospective Outlook on Long term Energy Systems |
| PowNet | Network-Constrained UC /ED Model |
| PRIMES | Price-Induced Market Equilibrium System |
| PSO | Particle Swarm Optimization |
| PV | Photovoltaic |
| Pyomo | Python Optimization Modelling Objects software |
| PyPSA | Python for Power Systems Analysis |
| QC | Quantum Computing |
| OF | Objective Function |
| QP | Quadratic Programming |
| RES | Renewable Energy Sources |
| RO | Robust Optimization |
| RTO | Regional Transmission Organization |
| RTS | Reliability Test Systems |
| SCED | Security Constrained Economic Dispatch |
| SCUC | Security Constrained Unit Commitment |
| SDG(s) | Sustainable Development Goal(s) |
| SF-OPF | Shift Factor Optimal Power Flow |
| SIDC | EU Single Intraday Coupling |
| SO | Stochastic Optimization |
| STLF | Short-Term Load Forecasting |
| SUC | Stochastic UC |
| TEMOA | Tools for Energy Model Optimization or Analysis |
| TEP | Transmission Expansion Planning |
| TIMES | Integrated MARKAL-EFOM System |
| UC | Unit Commitment |
| UCED | Unit Commitment and Economic Dispatch |
| VPP | Virtual Power Plant |
| VRE | Variable Renewable Energy |
| WASP | Wien Automatic System Planning |