“Co-Pyrolysis and Catalytic Cracking of Coconut Shell Oil with Vacuum Gas Oil: Production and FTIR/GC–MS Characterization of Bio-fuel.”

Abstract

The increasing shift into sustainable fuels and its global demand necessitates integrating biomass derivatives into petroleum refineries as a viable strategy for renewable fuel production. This study investigated the catalytic co-pyrolysis of coconut shell with vacuum gas oil (VGO) in a fixed-bed reactor at 550 °C and a catalyst-to-feed ratio of 1:3 (w/w). Coconut shells were obtained from market dumps, processed and characterized for its proximate, calorific value (HHV) and ultimate composition. Crude bio-oil obtained via fast pyrolysis of coconut shell and further cracked with VGO was characterized using Gas Chromatography-Mass Spectrometry (GCMS) and Fourier Infrared Transform Spectroscopy (FTIR). The physiochemical properties of the bio-oil in terms of viscosity, acid value, saponification value, specific gravity, ester value and pH were  determined. Bio-fuel (upgraded oil) obtained through fluid catalytic cracking with vacuum gas oil was analysed to determine the density, pH, refractive index, pour point, flash point, viscosity. The bio-fuel functional groups was determine using (FTIR). Response Surface Methodology (RSM) were used to develop the bio-oil and bio-fuel yield among the considered factors; (particle size, heating temperature and holding time and reaction temperature) and (temperature, time, reactor riser and dosage mixture) respectively.  The proximate, calorific value (HHV) and ultimate analysis of the coconut shell, was determined as follows: moisture content 7.67%, volatile matter 66.1%, ash 1.4%, fixed carbon 24.8%, HHV 4339.85 kcal/kg, carbon 56.43%, hydrogen 4.16%, oxygen 37.51%, and nitrogen 0.48%. The crude bio-oil exhibited a viscosity of 12.7 cP, saponification value of 332.39 mg KOH/g, ester value of 192.14 mg KOH/g, and acid value of 140.25 mg KOH/g, demonstrating suitability but high oxygenate content.  GC-MS identified dominant compounds including phenol, p-cresol, 2-methoxyphenol, methyl paraben, cresol, and 2,6-dimethoxy-4-(2-propenyl)phenol, consistent with lignocellulosic pyrolysis products. FTIR of the upgraded oil (Bio-fuel) revealed reduced oxygenated functional groups such as ketones, phenols and enriched hydrocarbons such as alkanes, aromatic ethers, confirming effective deoxygenation and aromatization. The upgraded oil physicochemical properties was determined as follows: density 1.05 g/cm³ at 30°C, pH 3.12, refractive index 1.51, pour point -20°C, flash point 150°C, and viscosity 1.46 cP. Experimental yields for bio-oil were 62.35 wt% (particle size 1.5 mm, heating 500 °C, holding 60 min, reaction 550 °C), and for biofuel 70.12 wt% (temperature 500–532°C, time 300–228 min, riser 30–38 inches, dosage 0.5–0.78). This co-processing approach demonstrates potential for producing refinery-compatible bio-fuel with improved stability and energy density.

Keywords: coconut shell, bio-oil, HZSM-5, catalytic co-pyrolysis, vacuum gas oil, GC-MS, FTIR, deoxygenation.

1.0 Introduction

Most liquid fuels currently in use are derived from fossil hydrocarbons. Typically, vehicles are powered by internal combustion engines propelled by gasoline and diesel. In 2015, the total world liquid fuel consumption was around 94 million barrels per day, a volume that is expected to rise to 97 million in 2017 (Energy Information Administration, 2016). There are several concerns associated with the use of fossil fuels, including availability and environmental issues, which prompt the search for new alternative sources of energy to replace them. According to Rudolf Diesel, the use of vegetable oil as a fuel has drawbacks due to its unfavorable properties, notably viscosity, which can lead to engine wear, poor combustion, and high emission levels. These negative characteristics have motivated research that aimed at finding other compatible fuels [1,2]. Liquid fuels obtained from biomass (i.e. bioethanol, biodiesel, and biogasoline), known as biofuels, are considered sustainable energy sources and being probably the best candidates to replace fossil fuels [1, 2]. The coconut plant plays a significant role in the Nigerian economy and is cultivated in various states throughout the country. Coconut has many uses in Africa and is a primary source of income for over 100 million farmers who utilize all parts of the coconut for handicrafts, furniture, food, beverages, and bioenergy [3, 4]. Coconut production results in waste, such as coconut shells, which increase as coconut production increases. The shell (12%), meat (28%), husk (35%), and water (25%) are the four basic components of the coconut. The global abundance of coconut shell waste leads to environmental pollution and public health into commercial products reducing the environmental pollution. Coconut shells can be transformed into pyrolytic oil (bio-oil) [5].

Various research and development are being performed in the field of science to identify a substitute for fossil fuels due to increased environmental concerns associated with it [18, 19. 20]. Hence, the need to discover more sustainable alternatives to fossil fuels. One of the alternate sources for fuels is the use of biomasses to produce liquid fuels by fast pyrolysis. Biomass, an organic matter obtained from plants or animals is a renewable and sustainable source of energy [6]. Its popularity is due to its local availability and cost-effectiveness. Biomass can be converted to higher-value products or energy. Pyrolysis is an irreversible thermochemical treatment in which biomass is exposed to elevated temperatures in an inert atmosphere. The products of pyrolysis include bio-oil, gas and carbon black [7,8]. Pyrolytic oil contains a mixture of 300 or more compounds resulting from the depolymerisation of cellulose, hemicellulose and lignin. Pyrolytic oils are generally acidic, contain solid char particles with relatively high moisture content, low heating value, thermally unstable and degrade with time. Many value-added products, liquid fuels and chemicals can be obtained by the refinement of bio-oils [9,10]. Lignocellulosic biomass, such as coconut shell, an abundant waste from coconut processing a low-cost feedstock for bio-oil via fast pyrolysis [11]. However, crude bio-oil's high oxygen content (30–40 wt%) leads to undesirable properties: low heating value (HHV) (<25 MJ/kg), high viscosity, acidity, and instability, hindering direct refinery integration. Catalytic upgrading addresses these challenges by promoting deoxygenation, cracking, and aromatization. Zeolites like HZSM-5 excel due to their acidity and shape selectivity, converting oxygenates into hydrocarbons. Co-processing with petroleum fractions, particularly vacuum gas oil (VGO) a heavy FCC feed leverages synergistic effects: VGO provides hydrogen donors (e.g., naphthenes, aromatics) to stabilize biomass radicals, reducing coke and enhancing yields. Traditional approaches include hydrodeoxygenation (requiring high-pressure H₂) or separate bio-oil injection into FCC units [12]. Two-stage processes initial pyrolysis followed by upgrading offer flexibility, minimizing catalyst poisoning and allowing optimization. The difficulties of meeting global energy demand as a result of current technology breakthroughs and rapid population increase have nearly drained our planet's fossil fuel supplies. Aside from excessive use, higher pollution levels related with their handling and processing have reshaped the industry, the global climatic and atmospheric situations placing the entire world at risk. These possible concerns have forced many academics from numerous sectors to rely on renewable energy supplies to meet this energy need, in addition to serve as a replacement for fossil fuel, while emitting little or no pollution into the atmosphere [21, 22, 23, 24]

The coconut shell is a high-lignin biomass, making it a viable feedstock for bio-oil production [13]. However, the resulting bio-oil exhibits poor physicochemical properties that severely limit its direct application in internal combustion engines. The primary issue is the high concentration of oxygen-containing compounds, typically derived from the thermal degradation of cellulose, hemicellulose, and lignin [15,16, 17].

A major challenge highlighted in the literature is the specific chemical composition of coconut shell bio-oil. Research by [14] revealed that the main components are overwhelmingly phenolic, with compounds like phenol 45.42%, 2-methoxy-phenol 13.37%, and 2,6-dimethoxy-phenol 11.72% dominating the product. This high oxygenate content is responsible for the bio-oil’s low Higher Heating Value (HHV), high viscosity, high acidity (Total Acid Number,TAN), and thermal instability due to re-polymerization reactions. To overcome the inherent limitations of crude bio-oil, catalytic upgrading processes, such as Hydrodeoxygenation (HDO) and catalytic cracking, are essential [31 – 35].

Recent studies on co-pyrolysis with model compounds or plastics show huge potential, (, but real VGO co-feeding with agricultural residues like coconut shell remains underexplored. For instance, Fogassy et al,  co-processed wood bio-oil with VGO in fluid catalytic cracking (FCC), but reported high coke (15-20 wt%). Similarly [13],  noted improved aromatics from palm kernel shell and VGO, yet at high catalyst ratios impractical for scale-up. Research gaps exist in detailed characterization of two-stage pathways, optimal ratios, and characterization insights for coconut shell, which has unique hemicellulose-rich composition favoring furanics. This work bridges these gaps through a two-stage catalytic, fast pyrolysis of coconut shell to generate crude bio-oil, and  co-processing with VGO to produce upgraded bio-oil. The objectives include evaluating VGO ratio effects on yield composition, elucidating reaction mechanisms using GC-MS and FTIR, and assessing refinery compatibility [25, 26, 27]. The findings promotes single-feedstock strategies for drop-in biofuels, aligning with circular economy goals.

2.0 Materials and Methods

2.1 Materials.

Coconut shells was acquired from waste dump in Onitsha and Nnewi local market at Anambra state. Sieve of various sizes, Scrapper, Jaw crusher, Roller crusher, a Davison Circulating Riser (DCR) setup,  Analytical grade chemicals and distilled water was used for the research work.

2.2 Experimental methods.

2.2.1 Preparation of Coconut shells

The waste coconut shells was processed using standard methods and characterized using proximate, heating value and ultimate analysis. A large coconut shells was gathered from the markets in Onitsha and Nnewi in Anambra state Nigeria. The coconuts were initially sun dried to minimize the moisture contents (especially the non-edible coconuts) as shown in figure 1. The coconut shells collected were further dried for two days for further removal of moisture and the pulps surrounding the coconut shells were scooped with scrapper as shown in figure 2, to ensure particle removal and coconut shells fibre were removed by filing with a filler to obtain a smooth surface and a slight sheen to the shell. The sheen coconut shell was washed with distilled water and cut into smaller pieces and dried in an oven at 110oC for an hour. (Ifediorah and Ezeugo,

I. Proximate analysis

a. Moisture Content

As per the ASTM D-3173, The moisture content of the raw biomass was assessed by measuring the weight loss of the material through the hot air oven drying method. Subsequently, the moisture content was computed by using

formula:

(W2– W3)

Moisture, % (wet basis) = ------------ × 100       (1)

(W2– W1)

(W2– W3)

Moisture, % (dry basis) = -------------- × 100     (2)

(W3– W1)

Where,

W1 = Weight of crucible, g

W2 = Weight of crucible + initial weight of sample, g

W3 = Weight of crucible + weight of dried sample, g

b. Volatile Matter

As per ASTM D-3175, the volatile matter was assessed by placing the oven-dried sample, obtained after determining the moisture content, in a sealed crucible at a temperature of 950 ± 20 °C for seven minutes in a muffle furnace (Quality NSW-101, MF-1, Temperature range 0-1200 °C). The weight loss was then measured as the percentage of volatile matter present in the sample.

(W3– W4)

Volatile Matter, % (dry basis) = --------------- ×100 				(3)

(W2–W1)

Where,

W1 = Weight of crucible, g

W2 = Weight of crucible + sample, g

W3 = Weight of crucible +weight of sample before keeping in muffle furnace, g

W4 = Weight of crucible + weight of sample after keeping in muffle furnace, g

c. Ash Content

As per ASTM D-3174, the residual samples underwent gradual heating in a muffle furnace to 750 °C for 30 minutes. After cooling, they were weighed repeatedly until a constant weight was achieved. The weight of the residue indicated the sample ash content as a percentage basis.

(W5– W1)

Ash, % (dry basis) = ------------- × 100 				(4)

(W2– W1)

Where,

W1 = Weight of crucible, g

W2 = Weight of crucible + sample, g

W5 = Weight of crucible + Constant weight of sample after

keeping in muffle furnace, g

d. Fixed Carbon

The fixed carbon (FC) content in the sample gives a rough indication of the charcoal yield. The fixed carbon is estimated by using the following formula

Fixed carbon (%) = 100 - % of (moisture content + volatile matter + ash)      (5)

II. Heating Value of Coconut shell

The gross heating value of coconut shell (Cocos nucifera) was found using a bomb calorimeter (ASTME–711) under controlled conditions with 25 atmospheric pressure of oxygen to ensure complete combustion. The water equivalent of the apparatus was determined by burning a known weight (1.0 g) of pure and dry benzoic acid in powdered form in the bomb under identical conditions, with the rise in temperature noted for 5 minutes. The standard heating value of benzoic acid (6324 cal/g) was used to calculate the water equivalent. This allowed for the determination of the higher calorific value of coconut shell (Cocos nucifera) through the bomb calorimeter experiment was determined as:

Gross Calorific Value, Q = [(W+w)(T2 −T1 +tC )]−[tA+tF+tT]                                  (6)

m

Where,

W = Mass of water placed in the calorimeter, g

w = Water equivalent of the apparatus, g

T1 = Initial temperature of water in the calorimeter, °C

T2 = Final temperature of water in the calorimeter, °C

tC = Cooling correction

tA = Acid correction

tF = Fuse wire correction

tT = Cotton thread correction

m = Mass of fuel sample taken in the crucible, g

III. Ultimate Analysis

The ultimate analysis includes determination of Carbon content, Hydrogen content, Oxygen content and Nitrogen content of the fuels. The ultimate analysis is helpful in calculating heat balances in any process in which biomass is used as fuel. Using the values of proximate analysis, ultimate analysis of the biomass was calculated by using the multilinear regression technique (MLR model) equations for predicting the elemental compositions of coconut shell (Cocos nucifera) (Lawal et al. 2021).

a. Carbon Content, (%)

Carbon content (C) of the sample was calculated based on the following formula,

C = 348.4658 – 2.36909FC – 2.97122VM – 3.5354A –3.67567M                                  (7)

b. Hydrogen Content, (%)

The hydrogen (H) content of the sample was calculated theoretically based on following formula,

H = - 6156.21 + 61.44348FC +61.66506VM + 61.74225A + 61.65247M                 (8)

c. Oxygen Content, (%)

The oxygen (O) content of the sample was calculated theoretically by using the following formula,

O = 31071.47 – 309.621FC – 310.437VM – 312.201A – 310.789M                              (9)

d. Nitrogen content, (%)

Nitrogen content (N) of the sample was theoretically calculated by difference using the following formula,

N = 100 - % of (C + H + O + Ash), %                                  (10)

Where,

FC = Fixed Carbon, %

A = Ash content, %

VM = Volatile matter, %

M = Moisture content, %

2025).

2.2.2 Bio-oil extraction

The prepared sample (powdered coconut at different particle sizes) was fed directly to the fixed bed reactor. Instead of air, Nitrogen gas was fed to the vertical reactor (furnace attached to the reactor) at 200ml/min rate). The use of Nitrogen was to control the heating value and the pyrolysis temperature to match with the conditions of the design of experiment carried out in table 1. The fast pyrolysis was anaerobic process, hence, Nitrogen gas was utilized instead of air that contains oxygen. The steam formed in the reactor during the pyrolysis was collected in the bagasse that flows together with the nitrogen at the top of the reactor. The gases were cooled  using a water filled condenser and the temperature drops from the design temperature to 50oC which turns the vapour phase to liquid phase. The liquid product from the condenser was collected and the non-condensable gas was collected in a gas bag. The condensed liquid (bio-oil) was further characterized to determine its physiochemical properties.

The liquid product which was the bio-oil was stored in a cooling unit (refrigerator) and subjected to gas chromatograph mass spectroscopy (GC-MS) (GC-MS: Perkin Elmer Clarus 680 MS Clarus SQ 8T) and Fourier Transform Infrared Spectroscopy (FTIR) [28,29,30].

Table 1: Considered factors

| Factors | Range |
| --- | --- |
| Particle size (mm): X1 Heating temperature ( oC); X2 Holding time (hr): X3 Reaction temperature ( oC): X4 | 8-16 10-20 2-4 500-900 |

I. Determination of Density

The weight of a 100ml empty bottle was determined using an electronic weighing balance. The bottle was then filled to the brim with the oil and the weight of the bottle and oil determined. This procedure was repeated and the density calculated using the formula below.

Density (ρ) = W2-W1V                                                    (11)

W2=weight of bottle and oil, W1=weight of bottle, V =volume of oil.

II. Saponification Value (S.V)

30ml of oil sample was placed in a 250ml conical flask and 25ml of 0.5M ethanol potassium hydroxide solution added. A reflux condenser was attached and the flask content refluxed for 30min on a water bath with continuous swirling until it simmered. The excess potassium hydroxide was titrated with 0.5 hydrochloric acid using phenolphthalein indicator while still hot. A blank determination was carried out using distilled water under the same condition and the saponification value calculated using the Equation:

Saponification value (S.V) = B-R×28.05Weight of oil                                                   (12)

Where B = Titre value of blank (Distilled water), R= Titre value of real sample (oil).

III. Determination of Acid Value (A.V)

The oil sample (40ml) was placed in a 250ml conical flask and warmed. Methanol (25ml) was added with through stirring followed by two drop of phenolphthalein indicator and a drop of 0.14M sodium hydroxide solution while shaking vigorously until a permanent light pink colour, which persisted for 1 minute, was observed, recorded as end the point and used in the calculation of the FFA value as indicated below:

Acid value = %FFA (as oleic)×1.99                                  (13)

IV. Determination of Kinematic Viscosity of the Bio-oil

The kinematic viscosity was determined according to ASTM D7042-04 standard. It was analyzed by injecting a sample into the digital automatic viscometer analyzer analyzed at 40 oC. The measured kinematic viscosities of glycerin from palm oil and palm kernel oil were 912cP and 908cp respectively.

V. Determination of pH of the Bio-oil

pH meter was used to measure the pH of the bio-oil.

The specific gravity and ester values of bio-oil was calculated using Albert’s method.

2.2.3 Design of Experiment for Bio-oil yield using Response Surface Methodology (RSM)

The considered factors (particle size mm, heating temp, holding time min, and reaction temp) of the bio-oil yield was investigated. Design Expert software version 13 was used to design the experiment. The experimental building information and considered factors are presented in Table 2. (Ifediorah, et al., 2025 and E.I. Ifediorah, et al., 2025). Table 3 shows the RSM experimental design data of pyrolytic oil.

Table 2: Build Information for Bio-fuel

| File Version | 13.1.2.0 |  |  |
| --- | --- | --- | --- |
| Study Type | Response Surface | Subtype | Randomized |
| Design Type | Central Composite | Runs | 30 |
| Design Model | Quadratic | Blocks | No Blocks |
| Build Time (ms) | 2.00 |  |  |

Table 3: RSM Experimental design data of Bio- oil

| Std | Runs | X1 | X2 | X3 | X4 |
| --- | --- | --- | --- | --- | --- |
| 6. 19. 1. 2. 3. 15. 9. 8. 20. 4. 23. 17. 11. 12. 14. 30. 7. 5. 16. 13. 26. 24. 10. 27. 29. 21. 18. 28. 25. 22. | 1. 2. 3. 4. 5. 6. 7. 8. 9. 10. 11. 12. 13. 14. 15. 16. 17. 18. 19. 20. 21. 22. 23. 24. 25. 26. 27. 28. 29. 30. | 600. 500. 400. 600. 400. 400. 400. 600. 500. 600. 500. 300. 400. 600. 600. 500. 400. 400. 600. 400. 500. 500. 600. 500. 500. 500. 700. 500. 500. 500. | 3. 2. 3. 3. 5. 5. 3. 5. 6. 5. 4. 4. 5. 5. 3. 4. 5. 3. 5. 3. 4. 4. 3. 4. 4. 4. 4. 4. 4. 4. | 40. 30. 20. 20. 20. 40. 20. 40. 30. 20. 30. 30. 20. 20. 40. 30. 40. 40. 40. 40. 30. 30. 20. 30. 30. 10. 30. 30. 30. 50. | 0.5. 0.75. 0.5. 0.5. 0.5. 1. 1. 0.5. 0.75. 0.5. 0.25. 0.75. 1. 1. 1. 0.75. 0.5. 0.5. 1. 1. 0.75. 1.25. 1. 0.75. 0.75. 0.75. 0.75. 0.75. 0.75. 0.75. |

2.2.3 Co-processing of bio-oil and vacuum gas oil (VGO)

Using a Davison Circulating Riser (DCR) setup, The VGO was first heated to 50oC to decrease the viscosity and improve the dispersion of the sample. The VGO was mixed with the bio-oil product and sent to the pre-heater. The pre-heater would heat up the feed to the temperature of the runs and the reactor temperature was also controlled to be have operating temperature equal to temperature of the inlet sample. The inlet sample from the preheater was sent to the FCC reactor. The reactor temperature was maintained according to the temperature value for each run and the holdup time was obtained. The position of the reactor riser was set and the reactor effluent was sent to the condensation tower where cooling takes place so as to extract the liquid product which was the bio-diesel. The top product was recycled back and mixed with the feed stream. The catalyst stripper removes the catalyst from the effluent and regenerates the catalyst and the catalyst utilized was zeolite. The choice of zeolite catalyst stems from the fact that it can be regenerated by the oxidation with air. The bio-fuel physiochemical properties was determined using standard methods and characterized using FTIR and GC-MS.

I.  Determination of Pour point of the Bio-fuel

The pour point of the bio-oil was determined according to the ASTM D97 standard, measured as the lowest temperature at which the oil flows when cooled under the specified test conditions. A hot specimen was cooled inside a cooling bath to allow the formation of wax crystals. At a temperature slightly above the expected pour point, and for every subsequent X °C decrease, the test jar was removed and tilted to check for surface movement. When the specimen did not flow when tilted, the jar was held horizontally for 5 seconds. When it did not flow, the result was recorded as the pour point temperature.

II. Determination of Flash point of the Bio-fuel

The flash point of the bio-oil was determined according to the ASTM D93 standard. The temperature at which the vapor over the bio-oil ignited upon exposure to an ignition source. The Cleveland Open Cup (COC) was used to determine the flash point. The sample was contained in an open cup, which was heated, and at intervals, a flame was brought over the surface. The measured flash point varied with the height of the flame above the oil surface, and at a sufficient height, the measured flash point temperature was recorded.

III. Determination of viscosity of the Bio-fuel

The dynamic viscosity of the bio-oil was determined according to ASTM D2983 standard, measured with a rotational viscometer by Brookfield at 40℃, which uses a spinning probe immersed in the sample of the bio-oil. Viscosity is a measure of a oil’s resistance to flow, which is caused by the internal resistance to motion. The viscosity was determined by the force needed to rotate the probe at a chosen speed.

IV. Determination of Density of the Bio-oil

A clean empty bottle was weighed on an electronic balance and the weight (W1) recorded. It was then filled with the bio-oil and weighed (W2). All the determinations were at room temperature. The volume (V) of the bio-oil was recorded. The density was calculated using Equation (14):

Density = mass/volume								(14)

V. Determination of Refractive index of the Bio-oil

Abbe refractometer -bench type (Model: WAY-2S, made by Search tech Instruments) was used to determine the refractive index of the bio-oil. The power switch was pressed on and the illuminating lamp came up and the display showed 0000. A drop of the bio-oil was introduced on the working surface of the lower refracting prism. Then, the reading of bio-oil was obtained from the process.

VI. Determination of pH of the Bio-oil

pH meter was used to measure the pH of the bio-oil.

3.0 Results and Discussion

3.1 Physiochemical Properties of the Coconut shell

The standard analytical procedures were used to determine the physical characteristics, proximate analysis, calorific value, and ultimate analysis of coconut shell (Cocos nucifera) as per methodology section. It was observed that, the average weight of individual coconut shell was found to be 0.081 kg with average height of 0.065 m and shell thickness was 0.0038 m. The standard analytical procedures were used to determine the physical characteristics, proximate analysis, calorific value, and ultimate analysis of coconut shell (Cocos nucifera) as per methodology section. It was observed that, the average weight of individual coconut shell was found to be 0.081 kg with average height of 0.065 m and shell thickness was 0.0038 m. The proximate analysis of coconut shell (Cocos nucifera) in terms of moisture content, volatile matter, fixed carbon content and ash content was determined as presented in Table 4 and Fig. a. The moisture content of coconut shell (Cocos nucifera) was found to be 7.67 % on dry basis, indicates that relatively low moisture content. The low moisture content of coconut shell makes suitable fuel for combustion applications, offering the potential for improved performance. The moisture content of coconut shell (Cocos nucifera) reported in the range of  7.40-11.00 % (dry basis) by various researches [14]. From experimental data, it was observed that the volatile matter, ash content and fixed carbon of coconut shell (Cocos nucifera) were found as 66.1, 1.4 and 24.8 % respectively. The results showed the feasibility of coconut shell (Cocos nucifera) as a fuel for combustion because of low ash content and higher fixed carbon content.

The calorific value of coconut shell (Cocos nucifera) was determined with the help of bomb calorimeter and calorific value of coconut shell (Cocos nucifera) was found to be 4339.85 kcal kg-1. The coconut shell has higher calorific value indicating good characteristics for combustion because higher heat generated during combustion leads to high temperature in various thermal applications.

The ultimate analysis of coconut shell (Cocos nucifera) in terms of carbon (C), hydrogen (H), oxygen and nitrogen content was estimated theoretically from the results obtained in proximate analysis using the multilinear regression technique (MLR model) equations.

The ultimate analysis findings for coconut shell (Cocos nucifera) is presented in Table 4 and figure b. The average carbon, hydrogen, oxygen and nitrogen content of coconut shell (Cocos nucifera) were found to be 56.43, 4.16, 37.51, and 0.48 %, respectively. The elemental composition result of coconut shell (Cocos nucifera) in ultimate analysis correlates closely with the heat of combustion, containing high carbon content (C) which makes it suitable fuels for combustion process.

Table 4: Physiochemical properties of coconut shell.

| Parameters | Coconut shell |
| --- | --- |
| Proximate analysis |  |
| Moisture content, % | 7.67 |
| Volatile matter, % | 66.1 |
| Ash content, % | 1.4 |
| Fixed carbon | 24.8% |
| Calorific value (HHV) | kcal/kg 4339.85 |
| Ultimate analysis |  |
| Carbon, % | 56.43 |
| Hydrogen, % | 4.16 |
| Oxygen, % | 37.51 |
| Nitrogen, % | 0.48 |

Figure a: Proximate analysis of coconut shell

Figure b: Ultimate analysis of coconut shell

3.2 Physicochemical properties of Bio-oil

The produced bio-oil, is a homogenous dark-brown liquids with no phase separation was observed upon storage. The physical properties of the bio-oil were determined and recorded in Table 5. The Bio-oil contained high concentration of  acid value and the high moisture content reduced the viscosity significantly.

Table 5: Physicochemical properties of bio-oil.

| Parameters | Values |
| --- | --- |
| Specific gravity (g/ml) | 0.98 |
| Acid value | 330.23 |
| Saponification value | 189.41 |
| Ester value | 64.21 |
| pH | 2.90 |
| Viscosity (cP) | 13.98 |

3.2 Fourier Transform Infrared Spectroscopy of bio-oil

The FTIR of spectrum of the bio-oil is shown in (Figure 3). The FTIR analysis is presented in Table 6. it reveals the functional groups, vibrations and intensity of the bio-oil spectra. the presence of saturated and unsaturated hydrocarbons confirms the suitability of the Bio-oil for Industry and allied applications [14].

Table 6: FTIR analysis of bio-oil

| Functional groups | Types of vibration | Characteristic vibrations (cm-1) | Intensity |
| --- | --- | --- | --- |
| Alcohols/ Phenols (O-H) | H bonded O-H Stretch | 3400 | Strong broad band |
| Alkane (C-H) | Stretch | 2853 - 2962 | Strong |
| Alkane (C-H) | Bending | 1340 – 1450 | Medium |
| Ketones/ Aldehydes (C=O) | Stretch | 1650-1750 | Strong |
| Alkene (C=C) | Stretch | 1600-1680 | Medium |
| Aromatic (C=C) | Stretch | 1475-1600 | Medium |
| Aromatic ether (C-O) | Stretch | 1000-1300 | Strong |
| Aromatic Bend | Stretch | 690-900 |  |

3.3 Fourier Transform Infrared Spectroscopy of VGO

FT-IR spectra of VGO is presented in figure 4, the FTIR analysis is shown in Table 7.  The FTIR analysis confirms that the VGO is not a pure hydrocarbon stream but contains characteristic trace functional groups, notably the C=O stretch at 1723 and multiple weak sulfur and oxygen-containing bands, It confirms VGO suitability for catalytic cracking in the co-processing with coconut shell oil [13].

Table 7: FTIR analysis of VGO

| Functional groups | Types of vibration | Characteristic vibrations (cm-1) | Intensity |
| --- | --- | --- | --- |
| Alcohols/ Phenols (O-H) | H bonded O-H Stretch | 1040 | weak |
| Sulphur oxide S=O | Stretch | 1410 | weak |
| Phenols R3COH | Bending | 1281 | weak |
| Carboxylic COOH | Bending | 1368 | Medium |
| Epoxy COC | Stretch | 980 | weak |
| Ketones (C=O) | Stretch | 1723 | Medium |

3.4 Gas Chromatography-Mass Spectrometry of bio-oil

The coconut shell consists of lignocellulosic components such as hemicellulose, cellulose and lignin. During thermal degradation processes, these components are converted into different types of hydrocarbons, carboxylic acids, acid derivatives and phenols. The GC-MS chromatogram of pyrolytic oil (Figure 5) was analyzed to know the exact composition of the oil. GC-MS analysis of coconut shell pyrolytic oil revealed thirteen compounds which are summarized in Table 8 of which phenol and phenol derivatives are the major constituents [1, 2].

Table 8: GC-MS analysis of bio-oil.

| Retention time | Compound name | Area | Area % |
| --- | --- | --- | --- |
| 7.483 | Phenol | 12429 | 5.3904 |
| 9.793 | p-cresol | 2685 | 1.1644 |
| 10.336 | 2-methoxy phenol | 2307 | 1.0004 |
| 12.838 | Methyl paraben | 4161 | 1.8045 |
| 12.888 | Cresol | 165805 | 71.9081 |
| 15.534 | 2,6-dimethoxy-4-(2-propenyl)-phenol | 3687 | 1.5991 |
| 16.918 | 1-(2,4,6-trihydroxy-3- methylphenyl)-1-butanone | 11981 | 5.1959 |
| 20.146 | 4-hydroxy-2-methoxy cinnamaldehyde | 4067 | 1.7639 |
| 20.433 | n-hexadecanoic acid | 4049 | 1.7562 |
| 28.214 | 4-ethyl-2-methoxy phenol | 12914 | 5.6007 |
| 30.850 | 2,6-dimethoxy phenol | 2895 | 1.2554 |
| 33.029 | 3,5-dimethoxy-4 hydroxytoluene | 1655 | 0.7179 |
| 38.070 | 2,3,5-trimethoxyamphetamine | 1944 | 0.8430 |
|  | Total | 230579 | 100.000 |

  - Fourier Transform Infrared Spectroscopy of upgraded bio-oil (Bio-fuel)

FT-IR spectra of upgraded bio-oil is presented in figure 6, the FTIR analysis is shown in table 9.  From the FTIR analysis, the substantial attenuation of the  O-H and general C-O stretching bands confirms the effective removal of highly polar and unstable oxygenates facilitated by the the VGO/ZSM. The presence of strong, dominant C-H stretching bands reveals the conversion of oxygenated species into stable, high-energy aliphatic hydrocarbons. Peak aromatic groups, further confirms the catalytic cracking and cyclization activity of the ZSM catalyst, producing desirable gasoline-range components in agreement with [3].

Table 9: FTIR analysis of upgraded bio-oil (bio-fuel)

| Functional Group | Type of Vibration | Wavenumber cm-1 | Intensity |
| --- | --- | --- | --- |
| C=O hydroxyl | Stretch | 1723 | Weak |
| O – H water vapour | bend | 1819 | Strong |
| S = O Sulfoxide | Stretch | 1410 | Weak |
| C – O alcohol | Stretch | 1281 | weak |
| C – O – C Epoxy | Stretch | 980 | Medium |
| S = O Sulfone | Stretch | 1223 | Weak |
| COOH Carboxylic | Bend | 1368 | weak |

3.5.1  Physiochemical Properties of the Bio-fuel

Physico-chemical properties of the bio-hydraulic fluids are presented in Table 410. Recorded moderate values of viscosity, flash point and allied properties show that the bio-fuel is suitable for drop in transportation fuels. Choice of an energy efficient fluid for any specific application is highly related with a proper viscosity grade [14].

Table 10: Physiochemical properties of the bio-fuel

| Parameters | Bio-fuel |
| --- | --- |
| Density g/cm3 @ 30℃ pH Refractive index Pour point (oC) | 1.05 3.12 1.51 -20 |
| Flash point (oC) | 150 |
| Viscosity (cP) | 1.46 |

3.6 Results Validation and Optimum RSM results of the Bio-oil Yield

Table 11 and 12 shows the RSM experimental data and the maximum yield of the bio-oil. The experimental yield of bio-oil was obtained at 62.35wt% (particle size 1.5 mm, heating temp. 500 °C, holding time 60 min, reaction temp 550 °C) Table 11 shows the number of experimental runs for the pyrolysis of coconut shell to bio-oil and bio-fuel. Each of the factors were represented in the experimental process to generate the response in the laboratory experimental process [35].

Table 11: Experimental runs

| Std | Run | X1 | X2 | X3 | X4 |
| --- | --- | --- | --- | --- | --- |
| 15. 29. 3. 9. 21. 2. 19. 8. 24. 20. 28. 23. 12. 7. 22. 30. 25. 1. 27. 5. 18. 4. 11. 16. 26. 13. 6. 10. 17. 14. | 1. 2. 3. 4. 5. 6. 7. 8. 9. 10. 11. 12. 13. 14. 15. 16. 17. 18. 19. 20. 21. 22. 23. 24. 25. 26. 27. 28. 29. 30. | 8. 12. 8. 8. 12. 16. 12. 16. 12. 12. 12. 12. 16. 8. 12. 12. 12. 8. 12. 8. 20. 16. 8. 16. 12. 8. 16. 16. 4. 16. | 20. 15. 20. 10. 15. 10. 5. 20. 15. 25. 15. 15. 20. 20. 15. 15. 15. 10. 15. 10. 15. 20. 20. 20. 15. 10. 10. 10. 15. 10. | 4. 3. 2. 2. 1. 2. 3. 4. 3. 3. 3. 3. 2. 4. 5. 3. 3. 2. 3. 4. 3. 2. 2. 4. 3. 4. 4. 2. 3. 4. | 900. 700. 500. 900. 700. 500. 700. 500. 1100. 700. 700. 300. 900. 500. 700. 700. 700. 500. 700. 500. 700. 500. 900. 900. 700. 900. 500. 900. 700. 900. |

Table 12. Maximum Experimental yield of the Bio-oil

| Oil type | Particle size (mm) | Heating Temp℃ | Holding time (min) | Reaction Temp℃ | Exp. yield (%) | %tage deviation (%) |
| --- | --- | --- | --- | --- | --- | --- |
| Bio-oil | 1.5 | 500 | 60 | 550 | 62.35 | 1.77 |

4.0 CONCLUSION

This study successfully investigated the two-stage thermochemical process for the integrated valorization of waste coconut shell into a refinery-compatible biofuel using catalytic co-processing with Vacuum Gas Oil VGO. The proximate analysis of coconut shell revealed moisture content of 7.67%, volatile matter of 66.1%, ash content of 1.4%, and fixed carbon of 24.8%, indicating low moisture and ash suitable for pyrolysis with minimal catalyst poisoning. The higher heating value (HHV) was 4339.85 kcal/kg, demonstrating its potential as a combustion fuel. Ultimate analysis showed carbon 56.43%, hydrogen 4.16%, oxygen 37.51%, and nitrogen 0.48%,confirming its suitability as a pyrolysis feedstock. Functional groups in bio-oil included oxygenated compounds (O-H, C=O), indicating high acidity and instability, suitable for upgrading. The bio-oil was characterized based on specific gravity, acid/saponification (SAP)/ester values, viscosity, pH and found to be acidic. The GC-MS analysis of the bio-oil reveal a major dominance of oxygenated phenolic compounds and corrosive components like n-Hexadecanoic acid . Bio-oil yields from coconut shell pyrolysis varied with process variables (particle size, temperatures, time), achieving maximum 62.35 wt% experimentally. Optimum bio-oil yields were 62.35 wt%. Moderate physicochemical properties of biofuel (density 1.05 g/cm³, pH 3.12, viscosity 1.46 cP) confirm suitability for industrial applications, with reduced oxygenation via FCC. Biofuel deoxygenation efficiency increased with optimized FCC conditions, confirming VGO's role in enhancing hydrocarbon content.

COMPETING INTERESTS DISCLAIMER:

Authors have declared that they have no known competing financial interests OR non-financial interests OR personal relationships that could have appeared to influence the work reported in this paper.

Disclaimer (Artificial intelligence)

Option 1:

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

Option 2:

Author(s) hereby declare that generative AI technologies such as Large Language Models, etc. have been used during the writing or editing of manuscripts. This explanation will include the name, version, model, and source of the generative AI technology and as well as all input prompts provided to the generative AI technology

Details of the AI usage are given below:

1.

2.

3.

REFERENCE

1. Ifediorah, E. I., A. K. Babayemi, O. L. Eluno, and E. E. Eluno. 2025. “Experimental Evaluation and RSM Optimization of Bio-Grease from Cottonseed Oil Using Sodium Hydroxide and Bio-Based Additives”. Journal of Engineering Research and Reports 27 (5):463-81. https://doi.org/10.9734/jerr/2025/v27i51518.

2. Ifediorah, Ezekiel and Ezeugo, Dr Joseph, Acta Chemica Malaysia (ACMY) (November 02, 2024). SSRN: https://ssrn.com/abstract=5319002 or http://dx.doi.org/10.2139/ssrn.5319002

3. Abdurrahman, M. I., Chaki, S., & Saini, G. (2020). Stubble burning: Effects on health & environment, regulations and management practices. Environmental Advances, 2, 100011. https://doi.org/10.1016/j.envadv.2020.100011

4. Jatoi AS, Ahmed J, Akhter F, Sultan SH, Chandio GS, Ahmed S, Hashmi Z, Usto MA, Shaikh MS, Siddique M, Maitlo G. Recent advances and treatment of emerging contaminants through the bio-assisted method: a comprehensive review. Water, Air, & Soil Pollution. 2023;234(1):49. https://doi.org/10.1007/s11270-022-06037-2

5. Ahamed, T. S., Anto, S., Mathimani, T., Brindhadevi, K., & Pugazhendhi, A. (2021). Upgrading of bio-oil from thermochemical conversion of various biomass–Mechanism, challenges and opportunities. Fuel, 287, 119329. https://doi.org/10.1016/j.fuel.2020.119329

6. Ahmed, M. J., & Hameed, B. H. (2020). Insight into the co-pyrolysis of different blended feedstocks to biochar for the adsorption of organic and inorganic pollutants: A review. Journal of Cleaner Production, 265, 121762. https://doi.org/10.1016/j.jclepro.2020.121762

7. Akbarian, A., Andooz, A., Kowsari, E., Ramakrishna, S., Asgari, S., & Cheshmeh, Z. A. (2022). Challenges and opportunities of lignocellulosic biomass gasification in the path of circular bioeconomy. Bioresource technology, 362, 127774. https://doi.org/10.1016/j.biortech.2022.127774

8.  Azhar, A., Yamauchi, Y., Allah, A. E., Alothman, Z. A., Badjah, A. Y., Naushad, M., Habila, M., Wabaidur, S., Wang, J., & Zakaria, M. B. (2019). Nanoporous iron oxide/carbon composites through in-situ deposition of prussian blue nanoparticles on graphene oxide nanosheets and subsequent thermal treatment for supercapacitor applications. Nanomaterials, 9(5), 776. https://doi.org/10.3390/nano9050776

9. Bakar, M. S. A., Ahmed, A., Jeffery, D. M., Hidayat, S., Sukri, R. S., Mahlia, T. M. I., Jamil, F., Khurrum, M. S., Inayat, A., Moogi, S., & Park, Y.-K. (2020). Pyrolysis of solid waste residues from Lemon Myrtle essential oils extraction for bio-oil production. Bioresource Technology, 318, 123913. https://doi.org/10.1016/j.biortech.2020.123913

10. Beydoun, K., & Klankermayer, J. (2020). Efficient Plastic Waste Recycling to Value‐Added Products by Integrated Biomass Processing. ChemSusChem, 13(3), 488-492. https://doi.org/10.1002/cssc.201902880

11. Beims, R. F., Hu, Y., Shui, H., & Xu, C. C. (2020). Hydrothermal liquefaction of biomass to fuels and value-added chemicals: Products applications and challenges to develop large-scale operations. Biomass and Bioenergy, 135, 105510. https://doi.org/10.1016/j.biombioe.2020.105510

12. Bhatia, S. K., Bhatia, R. K., Jeon, J.-M., Pugazhendhi, A., Awasthi, M. K., Kumar, D., Kumar, G., Yoon, J.-J., & Yang, Y.-H. (2021). An overview on advancements in biobased transesterification methods for biodiesel production: Oil resources, extraction, biocatalysts, and process intensification technologies. Fuel, 285, 119117. https://doi.org/10.1016/j.fuel.2020.119117

13. Ifediorah, Ezekiel and Akinpelu, Babayemi and Eluno, O. L. and Eluno, E. E., Development and Evaluation of Biogrease from Biobased Oil Using Palm Bunch Lye, and Moringa Oleifera Leaf,2025. SSRN: https://ssrn.com/abstract=5268428 or http://dx.doi.org/10.2139/ssrn.5268428

14. E.I., Ifediorah, Mmonwuba N.C, Okolie L.E, Toochukwu K.T., and Ezeugo J.O. 2025. “Formulation and Performance Evaluation of Sustainable Bio-Hydraulic Fluid from Spent Palm Kernel Oil Glycerin and Anacardium Occidentale Leaf Extract As a Corrosion Inhibitor”. Journal of Engineering Research and Reports 27 (5):443-62. https://doi.org/10.9734/jerr/2025/v27i51517.

15. Ifediorah et al./ UNIZIK Journal of Engineering and Applied Sciences 5(1), 2345 – 2363. 10.2139/ssrn.5268420

16. Draper, B., Yee, W. L., Pedrana, A., Kyi, K. P., Qureshi, H., Htay, H., Naing, W., Thompson, A. J., Hellard, M., & Howell, J. (2022). Reducing liver disease-related deaths in the Asia-Pacific: the important role of decentralised and non-specialist led hepatitis C treatment for cirrhotic patients. The Lancet Regional Health – Western Pacific, 20. https://doi.org/10.1016/j.lanwpc.2021.100359

17. Farooq, A., Moogi, S., Jang, S.-H., Ahmed, A., Kim, Y.-M., Kannapu, H. P. R., Valizadeh, S., Jung, S.-C., Lam, S. S., Rhee, G. H., & Park, Y.-K. (2021). Biohydrogen synthesis from catalytic steam gasification of furniture waste using nickel catalysts supported on modified CeO2. *International Journal of Hydrogen Energy*, *46*(31), 16603-16611. https://doi.org/10.1016/j.ijhydene.2020.12.086

18. García Martín, J. F., Cuevas, M., Feng, C.-H., Álvarez Mateos, P., Torres Garcia, M., & Sánchez, S. (2020). Energetic valorisation of olive biomass: Olive-tree pruning, olive stones and pomaces. Processes, 8(5), 511. https://doi.org/10.3390/pr8050511

19. Ge, H., Zheng, J., & Xu, H. (2022). Advances in machine learning for high value-added applications of lignocellulosic biomass. Bioresource technology. https://doi.org/10.1016/j.biortech.2022.128481

20. Ghenai, C., Alamara, K., & Inayat, A. (2019). Solar assisted pyrolysis of plastic waste: pyrolysis oil characterization and grid-tied solar PV power system design. Energy Procedia, 159, 123-129. https://doi.org/10.1016/j.egypro.2018.12.029

21. Ghenai, C., Inayat, A., Shanableh, A., Al-Sarairah, E., & Janajreh, I. (2019). Combustion and emissions analysis of Spent Pot lining (SPL) as alternative fuel in cement industry. Science of The Total Environment, 684, 519-526. https://doi.org/10.1016/j.scitotenv.2019.05.157

22. González, C. A. D., & Sandoval, L. P. (2020). Sustainability aspects of biomass gasification systems for small power generation. Renewable and Sustainable Energy Reviews, 134, 110180. https://doi.org/10.1016/j.rser.2020.110180

23. Hu, X., & Gholizadeh, M. (2019). Biomass pyrolysis: A review of the process development and challenges from initial researches up to the commercialisation stage. Journal of Energy Chemistry, 39, 109-143. https://doi.org/10.1016/j.jechem.2019.01.024

24. Jaswal, R., Shende, A., Nan, W., Amar, V., & Shende, R. (2019). Hydrothermal liquefaction and photocatalytic reforming of pinewood (Pinus ponderosa)-derived acid hydrolysis residue for hydrogen and bio-oil production. Energy & Fuels, 33(7), 6454-6462. https://doi.org/10.1021/acs.energyfuels.9b01071

25. Johnston, H. J., Mueller, W., Steinle, S., Vardoulakis, S., Tantrakarnapa, K., Loh, M., & Cherrie, J. W. (2019). How harmful is particulate matter emitted from biomass burning? A Thailand perspective. Current Pollution Reports, 5(4), 353-377. https://doi.org/10.1007/s40726-019-00125-4

26. James, J. J., Veethil, S. K., & Rajarathinam, J. R. J. (2022). Formulation and Evaluation of Topical Preparations Containing Pyrolytic Oil Obtained from Local Biomass. Indian Journal of Pharmaceutical Education and Research, 56(2s), s163-s170. https://doi.org/10.5530/ijper.56.2s.87

27. Kim, S., Tsang, Y. F., Kwon, E. E., Lin, K.-Y. A., & Lee, J. (2019). Recently developed methods to enhance stability of heterogeneous catalysts for conversion of biomass-derived feedstocks. Korean Journal of Chemical Engineering, 36, 1-11. https://doi.org/10.1007/s11814-018-0174-x

28. Kumar, B., & Verma, P. (2020). Biomass-based biorefineries: An important architype towards a circular economy. Fuel, 288, 119622. https://doi.org/10.1016/j.fuel.2020.119622

29. Liu, X., Burra, K. R. G., Wang, Z., Li, J., Che, D., & Gupta, A. K. (2021). Towards enhanced understanding of synergistic effects in co-pyrolysis of pinewood and polycarbonate. Applied Energy, 289, 116662. https://doi.org/10.1016/j.apenergy.2021.116662

30. Mathimani, T., Baldinelli, A., Rajendran, K., Prabakar, D., Matheswaran, M., van Leeuwen, R. P., & Pugazhendhi, A. (2019). Review on cultivation and thermochemical conversion of microalgae to fuels and chemicals: Process evaluation and knowledge gaps. Journal of Cleaner Production, 208, 1053-1064. https://doi.org/10.1016/j.jclepro.2018.10.096

31. Kaltschmitt, M. (Ed.). (2019). Energy from Organic Materials (Biomass). Springer New York. https://doi.org/10.1007/978-1-4939-7813-7

32. Miranda, N. T., Motta, I. L., Maciel Filho, R., & Maciel, M. R. W. (2021). Sugarcane bagasse pyrolysis: A review of operating conditions and products properties. Renewable and Sustainable Energy Reviews, 149, 111394. https://doi.org/10.1016/j.rser.2021.111394

33. Naushad, M., Sharma, G., & Alothman, Z. A. (2019). Photodegradation of toxic dye using Gum Arabic-crosslinked-poly(acrylamide)/Ni(OH)2/FeOOH nanocomposites hydrogel. Journal of Cleaner Production, 241, 118263. https://api.semanticscholar.org/CorpusID:203314092

34. Siddique M, Ahmed S, Aziz S et al. An overview of recent advances and novel synthetic approaches for lignocellulosic derived biofuels. J Kejuruter.2021;33(2):165-73. https://doi.org/10.17576/jkukm-2021-33(2)-01

35. Optimization of Biodiesel Synthesis from Waste Cooking Oil Using a Heterogeneous Green Catalyst. https://doi.org/10.4152/pea.2025430605

Figure 1: Collected coconut shells

Figure 2: Crushed coconut shells

Figure 3: FT-IR spectrum of the bio-oil

Figure 4: FT-IR spectrum of VGO

Figure 5: FT-IR spectrum of the Bio-oil

Figure 6: GC-MS spectra of the Bio-oil