An A-Guided Improved Ant Colony Optimization Algorithm for Mobile Robot Path Planning

ABSTRACT

To address the problems of slow convergence and susceptibility to local optima in traditional ant colony optimization (ACO) algorithms for global path planning of mobile robots, this paper proposes an improved ACO algorithm with three core methodological innovations: A*-guided differentiated pheromone initialization, a bidirectional A*-style heuristic factor, and strict line-of-sight (SLOS) path smoothing post-processing. Specifically, the proposed algorithm incorporates the A* algorithm to generate a prior path for targeted pheromone initialization, designs a bidirectional heuristic factor to strengthen global search guidance, and introduces a strict line-of-sight smoothing strategy to shorten the path while ensuring collision avoidance. Through simulation experiments on 20×20 and 30×30 grid maps, with the traditional ant colony algorithm as a benchmark for comparison, the results demonstrate that: the improved algorithm achieves an average runtime of only 0.67 seconds on the 20×20 map, representing a reduction of over 73.6% compared to the traditional algorithm; the average number of iterations decreases to 5.23, and the average path length is optimized to 29.36. The algorithm also maintains excellent performance on the 30×30 grid map with more complex environmental settings. This study validates that the proposed improvement strategies can significantly enhance the convergence speed, solution quality, and real-time performance of path planning. The proposed algorithm demonstrates broad practical applicability in mobile robot navigation. It provides an efficient and reliable path planning solution for intelligent robots deployed in intelligent manufacturing, autonomous warehouse handling, and unmanned patrol exploration.

Keywords: Ant Colony Optimization; A* Algorithm; Mobile Robot Path Planning; Heuristic function improvement; Global optimal path.

# INTRODUCTION

The rapid development of automation technology has continuously driven the accelerated expansion of robot applications across various fields [1]. In cutting-edge domains such as intelligent manufacturing scheduling, UAV airspace monitoring, satellite dynamic guidance, and autonomous robot exploration, trajectory planning accuracy has become a key indicator for evaluating system performance [2]. The core objective of path planning is to generate a collision-free and optimal route from a starting point to a destination for mobile robots under known environmental conditions [3].

Current research in robot path planning focuses on three main areas: shortest path planning [4], collision-free path generation [5], and dynamic planning with real-time capabilities [6]. Although Dijkstra's algorithm guarantees the global shortest path, it lacks directional guidance because it uses no heuristic information. This limitation significantly reduces its search efficiency [7]. The A* algorithm uses heuristic information to guide search direction while ensuring path optimality. However, in complex environments, its search space expands significantly as the number of nodes increases, leading to reduced computational efficiency [8]. While the RRT algorithm can efficiently obtain suboptimal solutions, its search efficiency deteriorates notably when pursuing the global optimal solution [9].

To overcome these limitations of traditional algorithms, researchers have turned to swarm intelligence approaches. These algorithms leverage biological collective collaboration mechanisms, offering robustness, distributed computing advantages, and superior exploration efficiency in complex path planning scenarios [10]. Inspired by the foraging behavior of ants in nature, Marco Dorigo proposed the Ant Colony Optimization (ACO) algorithm in 1992 [11], introducing a novel swarm intelligence paradigm to the field of metaheuristic optimization. Although ACO adapts well to path planning tasks, it faces several challenges: high computational costs, slow convergence, and premature convergence leading to search stagnation. These issues stem from the positive feedback mechanism of pheromones [12].

Consequently, performance improvement and convergence efficiency enhancement of ACO algorithms have become critical scientific issues urgently requiring resolution in the field of robot path planning [13]. As a typical NP-hard problem in the optimization domain, path planning's multi-objective nature precludes the direct acquisition of optimal solutions within polynomial time; feasibility of a given solution can only be verified through polynomial algorithms [14].

To address these shortcomings, Zheng Zhang et al. [15] proposed a hybrid framework named IACO-QDWA. This approach combines improved ant colony optimization (IACO) for global planning with a Q-learning-enhanced dynamic window approach (QDWA) for obstacle avoidance. Their method effectively solves three key problems: slow convergence, susceptibility to local optima, and delayed obstacle avoidance responses in dynamic environments. Similarly, Mingze Li et al. [16] developed an optimized APF-ACO algorithm for maritime applications. Their method constructs a COLREGs-compliant collision avoidance model and enhances both the artificial potential field (APF) method and the ACO algorithm. This approach solves collision avoidance and path planning for ships encountering static and dynamic obstacles, while overcoming the local optimum problem of conventional APF. Haixia Wang et al. [17] also developed an improved ACO algorithm for logistics robots. Their approach introduces artificial potential field gravitational forces to enhance the heuristic function, designs adaptive pheromone update rules, and applies triangular pruning for path smoothing. This method effectively addresses slow convergence, local optimality, and excessive turning points in 2D grid-based path planning.Tomitagawa et al. proposed an improved ant colony optimization (ACO) for energy-optimal path planning of indoor garbage collection robots under limited energy constraints. By jointly using path distance and garbage weight as heuristic information (visibility) and designing a multi-objective pheromone update mechanism based on travel distance and energy consumption, simulation results verify that the algorithm optimizes energy consumption while effectively controlling travel distance, significantly outperforming conventional and improved ACO algorithms that only consider a single factor [18].Yun Chen et al. introduced a path planning method for intelligent warehousing robots based on an improved ant colony algorithm. It establishes a three-color grid map to simulate unknown disturbances, optimizes the pheromone update mechanism, and accounts for path safety, length, and turning costs, while performing node removal and smoothing on the planned path. This method resolves blind search, deadlock, and slow convergence in traditional ACO for warehousing robot path planning, improving safety and accuracy [19].Pan Wu et al. proposed a two-level vehicle path planning model for multi-warehouse robots that integrates static and dynamic planning. In the static stage, a blocking factor is introduced to improve the ant colony optimization (ACO) algorithm to avoid congested nodes; in the dynamic stage, a dynamic priority mechanism is designed to adjust path strategies in real time. This model resolves various conflicts during multi-robot operation, improving efficiency and reducing operational costs [20].Yuanao Li et al. presented an improved ant colony optimization algorithm (Improved-ACO). It optimizes heuristic information calculation, introduces an enhanced Tanh function, adopts novel pheromone update and diffusion strategies, and combines a collision avoidance system with an improved B-spline curve for path smoothing. The algorithm addresses slow convergence, local optimality, and incompatibility with robot kinematic constraints in traditional ACO, enhancing the efficiency, smoothness, and adaptability of mobile robot path planning [21].Yanli Chen et al. proposed a hybrid path planning algorithm combining improved ant colony optimization and artificial potential field. By introducing a dynamic early-warning step adjustment strategy and an equipotential outer tangent circle escape method, the algorithm solves the problems of slow global path planning convergence, unreachable targets, and local minima in local obstacle avoidance for unmanned surface vehicles in complex dynamic environments, and achieves effective avoidance of both static and dynamic unknown obstacles [22].

Although these studies improve ACO through various strategies, three limitations persist. First, initial pheromones lack prior knowledge of global optima, resulting in blind early-stage search. Second, heuristic functions rely solely on unidirectional distance estimation, providing weak global guidance. Third, path smoothing often lacks rigorous safety verification, potentially allowing paths to traverse obstacles.These issues make it difficult for algorithms to balance convergence speed and solution quality in complex environments.

To address these deficiencies, this paper proposes an A*-guided improved ant colony algorithm (A-ACO). Specifically, we utilize A* prior paths to guide differentiated pheromone initialization. By employing globally optimal paths from A* as prior knowledge, we assign high initial pheromone levels to guide the search. This approach substantially reduces search blindness and enhances convergence efficiency. Second, we design a bidirectional A*-style heuristic function. This function considers distances from nodes to both start and goal points, significantly strengthening global guidance and improving search direction accuracy. Third, we introduce a strict line-of-sight (SLOS) safety smoothing strategy. This strategy ensures collision-free paths through dense sampling point verification. It effectively reduces path length and turning points while guaranteeing robot motion feasibility. Furthermore, elitist path differentiated pheromone enhancement and dynamic neighborhood pruning mechanisms are incorporated to better balance global exploration and local exploitation capabilities.

# 2. Traditional Ant Colony Algorithm

## 2.1 Node Transition Probability

In the traditional Ant Colony Optimization algorithm, each ant makes path construction decisions based on pheromone concentrations and heuristic information between nodes. The transition probability for the k-th ant moving from node i to node j at time t is defined as follows:

Pijkt=τijtα⋅ηijtβrϵΩkτirtα⋅ηirtβ,  &j∈Ωk0,  &j∉Ωk#2-1

where the heuristic function is defined as the reciprocal of the Euclidean distance between nodes:

ηijt=1δij#2-2

In the formulas, τijt denotes the pheromone intensity between node i and j at time t; ηijt represents the heuristic function value, characterizing the heuristic desirability of node transition; δij indicates the Euclidean distance between node i and j; parameters α and β respectively control the relative weights of pheromone and heuristic information; and Ωk represents the set of reachable successor nodes for ant k at current node i.

## 2.2 Pheromone Update Mechanism

After each iteration is completed, the pheromone on each edge (or path segment) is updated according to the following rules:

τijt+1=1-ρ⋅τijt+∆τijt#2-3

The global pheromone increment consists of the sum of pheromones released by all ants:

∆τijt=k=1MΔτijkt#2-4

The pheromone contribution of a single ant is inversely proportional to the quality of the path it constructs:

Δτijkt=Q/Lk,  &if ant k passes through edge(i,j)0,  &otherwise#2-5

Where: ρ denotes the pheromone evaporation rate, with 0<ρ<1 ; Q is a constant representing the pheromone intensity; the path length Lk is calculated by accumulating the distances δij​ between consecutive nodes on the path; and M is the ant colony size. This update mechanism enables shorter paths to accumulate more pheromones, thereby guiding subsequent ants to tend to select optimal paths.

# 3. Improved Ant Colony Algorithm

Aiming at the problems of slow convergence, excessive redundant turns in paths, and susceptibility to local optimality of the traditional ant colony algorithm in grid-based path planning, this paper proposes an improved A*-guided ant colony optimization algorithm (A*-ACO) through improvements in pheromone initialization, heuristic function design, path post-processing, and other aspects.

## 3.1A*-Guided Intelligent Pheromone Initialization

The traditional ACO algorithm employs global uniform pheromone initialization with τij0=C, which lacks prior guidance. This results in considerable blindness in the initial search of the ant colony, slow convergence speed, and particularly a large number of invalid searches in complex grid maps.

The A* algorithm is utilized to search for a reference path from the start point S to the end point E in the grid map, with its node sequence denoted as NA*={n1,n2,⋯,nL} (where n1​=S and nL​=E). The corresponding path edge sequence is defined as πA*={(n1,n2),(n2,n3),⋯,(nL-1,nL)}, representing the set of edges formed by consecutive nodes along the path.

This path is regarded as the "prior optimal path" for ant colony search, and differential pheromone initialization is performed: edges on the A* path are assigned a high pheromone value, while non-A* path edges are given a base pheromone value. This enables the ant colony to search towards the global optimal direction from the initial stage, significantly reducing search blindness, improving algorithm convergence speed, and decreasing the number of invalid iterations.

The fundamental pheromone initialization formula is formulated as follows:

τijt=τbase+ΔτA*,  &if(i,j)∈πA*τbase,  &otherwise#3-1

where τbase represents the baseline pheromone concentration (corresponding to τbase=2 in the implementation), and ΔτA* denotes the pheromone enhancement quantity for the A*  path (corresponding to ΔτA*=13 in the implementation, yielding an initial value of 15). The condition (i,j)∈πA* indicates that the node pair (i,j) constitutes a consecutive adjacent connection within the A* prior path, where πA* represents the edge sequence of the A* prior path (the set of edges formed by consecutive nodes).

By incorporating the prior knowledge from the A* algorithm, the pheromone initialization is upgraded from "unguided uniform distribution" to "guided differential distribution". This endows the ant colony with a clearer initial search direction, significantly improving the algorithm's convergence speed and reducing the number of invalid iterations.

## 3.2 Design of Bidirectional A*-Style Heuristic Factor

In traditional ant colony optimization (ACO), the heuristic factor ηij is usually constructed only based on the unidirectional distance from node j to the destination E (e.g., ηij=1/d(j,E)). Such a design fails to consider the bidirectional correlation of node j with both the start S and the destination E, resulting in weak guidance of heuristic information and making the ant colony search prone to deviating from the optimal path.

To address this limitation, we adopt a bidirectional A*-inspired heuristic strategy. This strategy combines the actual cost from start to current node with the estimated cost from current node to destination. The resulting heuristic function provides stronger global guidance. For any node j, its A* evaluation function is defined as the sum of the distance from the start S to node j and the heuristic distance from node j to the destination E. The heuristic factor ηij​ is set to be inversely proportional to this evaluation function. This mechanism enables ants to consider not only the local cost of the current transition but also the proximity to the destination during path selection, effectively reducing detours and improving search efficiency.

Based on the above idea, the traditional heuristic function is redefined and replaced as follows.

Basic version:

ηij=1γ1⋅dSj+γ2⋅djE+ε#3-2

Enhanced version:

ηij=1γ1⋅dSj+γ2⋅djE+ελ#3-3

dSj denotes the Euclidean distance from node j to the start S;

djE denotes the Euclidean distance from node j to the destination E;

γ1,γ2 are weight coefficients for the bidirectional heuristic terms;

ε is a small positive constant to avoid division by zero;

λ is the distance decay exponent.

The proposed formulation breaks through the limitation of traditional unidirectional heuristic factors by embedding bidirectional guidance (toward both start and destination) into the heuristic information. It better matches the global objective of minimizing path cost, improves the guidance accuracy of the heuristic factor, reduces invalid node selections during ant exploration, and further accelerates the convergence rate of the algorithm.

## 3.3 Strict Line-of-Sight (SLOS) Path Safety Smoothing

ACO-generated paths in discrete grids often exhibit zigzag turns. These turns are suboptimal and challenging for robot trajectory tracking. Traditional smoothing methods may also allow paths to cut corners through obstacles. To address this issue, this algorithm introduces Strict Line of Sight (SLOS) detection to ensure that the smoothed path is physically feasible.

A safe smoothing strategy combining strict line-of-sight detection and greedy shortcut search is proposed. Based on the original optimal path, starting from the start node, the algorithm searches backward from each node toward the destination. Strict line-of-sight detection is used to judge whether an obstacle-free straight connection (line of sight) exists between two nodes. If such a line of sight exists, the two nodes are connected directly (skipping all intermediate redundant nodes) to form a shortcut; otherwise, the search proceeds to the next node until all nodes are traversed. Finally, a smooth path without redundancy and collisions is obtained.

Strict Line-of-Sight Criterion

Let the coordinates of nodes n1 ​and n2​ be (x1,y1) and (x2,y2), respectively. Define the sampling point set:

Ρ=x1+tx2-x1,y1+ty2-y1t=sN,s=0,1,⋯,N#3-4

where the number of sampling points is

N=10⋅x1-x22+y1-y22#3-5

The strict line-of-sight condition is defined as:

SLOSn1,n2=true,  &if∀x,y∈P,Groundy,roundx=0false,  &otherwise#3-6

where G denotes the grid map matrix.

The smoothed path is denoted as:

πsmooth=nk1,nk2,⋯,nkm#3-7

where the index sequence satisfies:ki+1=maxj>kiSLOS(nki,nj)=true, k1=1

This post-processing step minimizes path length and the number of turns while guaranteeing path safety (collision-free). The length of the smoothed path satisfies  Lsmooth≤Loriginal.

## 3.4 Differentiated Pheromone Enhancement Mechanism for Strong Elite Paths

Traditional ACO typically uses either global uniform pheromone updates (all paths release pheromone equally) or simple elite bonuses (elite paths release fixed multiples). These schemes lack targeted reinforcement of the globally optimal path. Consequently, pheromone accumulates slowly, and locally optimal paths often dominate, causing algorithm stagnation.

To tackle this issue, a differentiated pheromone update mechanism is designed, combining conventional update for ordinary paths and double-strength enhancement for elite paths. The pheromone update is divided into two components: the conventional pheromone release from all feasible paths in the current iteration, and double pheromone release from the globally optimal elite path found so far. This allows pheromone on the elite path to accumulate much faster, forming strong heuristic guidance, suppressing the influence of locally optimal paths, and driving the algorithm to converge toward the global optimum.

Standard Pheromone Update Rule

τijt+1=1-ρ⋅τijt+∆τijt#3-8

The total pheromone increment is decomposed as:

∆τijt=m=1M∆τijmt+ωelite⋅∆τijbestt#3-9

where M denotes the size of the ant colony.

Contribution from the m-th ant (ordinary path):

∆τijmt=QLm,  &ifi,j∈πm0,  &otherwise#3-10

where Lm​ is the length of the path constructed by the m-th ant at iteration t.

Contribution from the globally optimal elite path:

∆τijbestt=QLbest,  &ifi,j∈πbest0,  &otherwise#3-11

where Lbest is the length of the globally optimal elite path found by the algorithm up to time t, and πbest denotes the edge sequence of the globally optimal elite path.

Differential weight: ωelite=2 meaning the contribution weight of the elite path is twice that of an ordinary ant.

By reinforcing the elite strategy, the algorithm can quickly focus on neighborhoods of high-quality solutions, avoiding interference from pheromone residues on inferior paths, and significantly improving convergence accuracy.

## 3.5 Dynamic Neighborhood Pruning Optimization Based on Destination Distance

Traditional ACO explores all 8 neighboring nodes without screening. When many invalid nodes lie far from the destination, computational costs increase and search efficiency deteriorates. Meanwhile, full-range exploration tends to lead ants to select nodes deviating from the destination, resulting in longer path lengths.

To overcome these drawbacks, a dynamic neighborhood pruning strategy based on destination distance is proposed. For each feasible 8-neighbor node of the current node (passable and unvisited), the nodes are sorted in ascending order according to their Euclidean distance d(j,E) to the destination E. Only the first K nodes closest to the destination (in this paper, K=5) are retained as the effective neighborhood, while invalid nodes far from the destination are pruned. In this way, ants only perform probabilistic selection within the effective neighborhood, which reduces computational cost and improves selection effectiveness.

Candidate Neighborhood Generation

where:

Neighbori denotes the physical 8-neighborhood of node i;

Tabuj is the tabu marker: Tabuj=0 indicates node j is unvisited, and Tabuj=1 means node j is already in the tabu list (no repeated access allowed);

Passi,j is the passability judgment function between nodes i and j.

Passi,j=1(passable) if there is no obstacle between i and j and robot motion constraints are satisfied (e.g., grid state is 0);

Passi,j=0 (impassable) if an obstacle exists between i and j (grid state is 1) or the node is outside the map boundary.

This dynamic neighborhood pruning optimization achieves improved search efficiency and enhanced destination guidance simultaneously. By pruning invalid neighboring nodes far from the destination, the search range is reduced from full 8-neighborhood to 5 effective neighbors, which effectively lowers the algorithm’s time complexity and makes it more suitable for path planning in large-scale complex grid maps. Meanwhile, retaining only the nearest valid nodes toward the destination ensures that each node selection by the ant colony moves closer to the goal, greatly reducing unnecessary detours. This optimizes the search process, further shortens the path length, and improves the overall quality of path planning.

## 3.6 Steps of the Improved Algorithm

Specific Steps of the Improved Algorithm:

Step 1

The working environment of the mobile robot is modeled using the grid method. The positions of the start point and the destination are determined, and obstacle regions (grid state set to 1) and passable regions (grid state set to 0) are marked, providing a digital environmental foundation for subsequent path search.

Step 2

Key parameters of the algorithm are initialized, including the ant colony size, pheromone evaporation coefficient ρ, pheromone weighting factor α, heuristic function weighting factor β, constant Q, and maximum number of iterations. The A* algorithm is applied to compute a prior path from the start point to the destination. A high initial pheromone value (set to 15) is assigned to edges along this prior path, while all other edges are initialized with a low pheromone value (set to 2), completing the differentiated pheromone initialization.

Step 3

All ants are placed at the start node, and the start node is added to the tabu list to prevent repeated visits by ants.

Step 4

The state transition probability of each ant is calculated using the bidirectional A-based heuristic factor*. Combined with the dynamic neighborhood pruning strategy (selecting the 5 feasible neighbors closest to the destination), the next node is selected via the roulette wheel method. The visited node is added to the tabu list. This process is repeated until the ant reaches the destination, completing a single path search.

Step 5

Record the path length, path node sequence, and number of turning points for each ant in the current iteration.

Step 6

If all ants have completed their searches in the current iteration, proceed to Step 7; otherwise, return to Step 4 to continue path construction for the remaining ants.

Step 7

Pheromone evaporation and update are performed according to the global pheromone update rule (incorporating the enhanced elite differential reinforcement mechanism, where an additional pheromone increment is superimposed on the globally optimal path). The tabu lists of all ants are cleared.

Step 8

If the current iteration number reaches the preset maximum number of iterations, or if no significant improvement in path length and quality is observed over several consecutive iterations, perform strict-line-of-sight (SLOS) based safe smoothing post-processing on the optimal path (redundant nodes are pruned and feasible straight connections are established via line-of-sight checks). The final optimized path is then output and the search terminates. Otherwise, return to Step 4 for the next iteration.

# 4. EXPERIMENTAL RESULTS AND ANALYSIS

## 4.1 Environment Modeling

Environment modeling serves as the foundation of path planning, whose core lies in abstracting a complex physical environment into a mathematically or digitally tractable model for machines. Its accuracy and adaptability directly determine the efficiency, safety, and feasibility of path planning.

Among various modeling approaches, the grid method is the most widely adopted. It partitions the environment into equally spaced cells and represents information using binary state flags (0 for free space, 1 for obstacles). With a simple structure and ease of implementation, it is suitable for path planning of various carriers such as unmanned aerial vehicles, wheeled mobile robots, and automated guided vehicles (AGVs). Therefore, this paper employs the grid method for map modeling.

The grid method is a typical global path planning modeling technique, whose core idea is to discretize the mobile environment into grids, thereby transforming the robot path planning problem into a connectivity problem between grid nodes [24]. The essence of grid-based modeling is to divide the environment into regular meshes, where each cell represents a discrete spatial region and can be labeled as free space, obstacle, or other states according to its properties. If an obstacle does not fully occupy a grid cell, the cell is still treated as fully occupied. In simulations, free grids are denoted by '0' and obstacle grids by '1', abstracting the environment map into a 2D matrix G that can be conveniently processed by programs. To prevent collisions between the robot and obstacles during motion, grids containing irregular obstacles are uniformly marked as occupied. Furthermore, the obstacle contour is expanded outward based on the physical radius of the robot to form an inflated obstacle region, which sets a safety boundary. This allows the robot to be simplified as a mass point model, while taking into account both its actual size and motion safety.

## 4.2 Results and Analysis

To verify the performance of the A*-guided improved ant colony optimization algorithm for path planning in grid maps, simulation experiments are conducted using MATLAB 2025.

### 4.2.1 Experimental Verification on 20×20 Grid Map

The experimental environment is a 20×20 grid map with approximately 50 obstacles configured. The key algorithm parameters are set as follows: maximum number of iterations = 100, ant colony size = 50, α=1, β=7, ρ=0.9, and Q=1.

We performed ten simulation experiments comparing the three algorithms. The proposed ACO algorithm demonstrates significant advantages in search efficiency, convergence speed, and path optimization. All core performance indicators achieve optimal results. Specifically, its average running time is 0.67 seconds. This represents a 73.6% reduction compared to the traditional algorithm and outperforms the 0.71 seconds reported in existing literature [13]. The average number of iterations is 5.23, significantly lower than 8.47 (literature algorithm [13]) and 14.47 (traditional algorithm). This demonstrates that our heuristic factor and pheromone initialization strategy effectively guide ant search directions. The average path length is 29.36, outperforming both the literature algorithm (30.40) and the traditional algorithm (30.62). This confirms the improved algorithm's ability to find higher-quality paths.From the perspective of convergence curves, the improved algorithm can rapidly converge to a suboptimal path in the early iterations, avoiding the problem of the traditional algorithm being prone to falling into local optima. Meanwhile, by virtue of A* path-based pheromone initialization and path smoothing processing, the path length is further shortened and the path safety is significantly improved.

The specific effects and results are illustrated in Figs. 1, 2, 3 and Table 1.

Fig. 1 Path simulation and convergence curve obtained by the traditional algorithm in the 20×20 grid environment

Fig. 2 Path simulation and convergence curve of Ref. [13] algorithm in 20×20 environment

Fig. 3 Path simulation and convergence curve of the proposed algorithm in the 20×20 environment

Table 1. Simulation comparison of three algorithms in 20×20 environment

| Algorithm | Average Running Time (s) | Average Path Length | Average Number of Iterations |
| --- | --- | --- | --- |
| Traditional ACO | 2.54 | 30.62 | 14.47 |
| ACO in Ref. [13] | 0.71 | 30.40 | 8.47 |
| Improved ACO | 0.67 | 29.36 | 5.23 |

### 4.2.2 Experimental Verification on 30×30 Grid Map

To further validate scalability, we conducted 30 simulation experiments on a larger 30×30 grid map using the same parameters.

On the 30×30 grid map, A-ACO maintains the same configuration as in the 20×20 map. After 30 trials, it retains performance advantages and demonstrates strong adaptability to larger-scale scenarios. All three core indicators remain optimal: the average running time is 1.06 seconds, a substantial reduction compared to the traditional algorithm and 22.76% faster than the literature algorithm [13]; the average number of iterations is only 9.57, which is much lower than those of the comparative algorithms.

This map contains 125% more grids with more widely distributed obstacles, imposing higher computational demands. Without prior guidance or neighborhood screening, the traditional algorithm suffers from exponentially increased running time, oscillatory convergence, and redundant paths.Although the literature algorithm has been improved, it exhibits a performance attenuation rate of 95.01% and insufficient path smoothness.

By virtue of strategies such as differentiated pheromone initialization and dynamic neighborhood pruning, A-ACO achieves a performance attenuation rate of only 59.61%, with stable convergence curves and convergence within 10 iterations in all tests, showing remarkable adaptability to large-scale complex environments.

The specific effects and results are illustrated in Figs. 4, 5, 6 and Table 2.

Fig. 4 Path simulation and convergence curve of the traditional algorithm in the 30×30 environment

Fig. 5 Path simulation and convergence curve of the algorithm in Ref. [13] in the 30×30 environment

Fig. 6 Path simulation and convergence curve of the proposed algorithm in the 30×30 environment

Table 2. Simulation comparison of three algorithms in 30×30 environment

| Algorithm | Average Running Time | Average Path Length | Average Number of Iterations |
| --- | --- | --- | --- |
| Traditional ACO | 12.38 | 44.48 | 54.97 |
| ACO in Ref. [13] | 1.38 | 42.82 | 14.33 |
| Improved ACO | 1.06 | 41.72 | 9.57 |

# 5. CONCLUSION

This paper proposes an A*-guided improved ant colony optimization algorithm (A*-ACO) to address three key challenges in mobile robot path planning: slow convergence, susceptibility to local optima, and path redundancy. Through five optimization strategies—differentiated pheromone initialization, bidirectional heuristic design, strict line-of-sight smoothing, elite path enhancement, and dynamic neighborhood pruning—we construct an efficient, stable, and practical path planning scheme. This study achieves full-process improvement, solves the inherent defects of the algorithm, and significantly enhances its engineering application capability, providing a new approach for robot navigation in complex scenarios.

The proposed algorithm demonstrates clear application value in scenarios such as AGV scheduling in intelligent manufacturing workshops, automated warehousing and logistics, and unmanned patrol inspection. In intelligent warehousing, for example, the algorithm can run on embedded industrial computers (e.g., NVIDIA Jetson) to receive commands from Warehouse Management Systems (WMS) and perform real-time path planning for material handling. In intelligent manufacturing production lines, it can be integrated with the Manufacturing Execution System (MES) to support multi-AGV collaborative scheduling and path optimization. Furthermore, regarding the computational performance on actual hardware platforms, this study strictly controlled the time complexity in simulation experiments (achieving an average runtime of only 0.67 seconds on 20×20 maps and 1.07 seconds on 30×30 maps), ensuring that it meets the basic real-time requirements of industrial applications (typically requiring response times of less than 1 second). The algorithm possesses the computational efficiency for real-time operation on standard embedded computing platforms, laying a theoretical foundation for engineering deployment.

Future work will extend this approach to dynamic environments with real-time obstacle avoidance and multi-robot cooperative planning. Additionally, we will enhance algorithm robustness against sensor noise in extreme environments (e.g., high temperature, dust) to improve deployment reliability. We will also conduct physical verification on specific platforms (e.g., differential drive robots) and address engineering challenges including real-time obstacle avoidance and multi-robot conflict resolution. This will facilitate the transition from simulation to practical deployment.

Disclaimer (Artificial intelligence)

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

# REFERENCES

[1] Antonelli, G., Fossen, T. I., & Yoerger, D. R. (2008). Underwater robotics. In Springer handbook of robotics (pp. 987-1008). Springer, Berlin, Heidelberg.

[2] Jin, Qibing, Chuning Tang, and Wu Cai. "Research on dynamic path planning based on the fusion algorithm of improved ant colony optimization and rolling window method." IEEE access 10 (2021): 28322-28332.

[3] Cherni, F., Boujelben, M., Jaiem, L., Boutereaa, Y., Rekik, C., & Derbel, N. (2017). Autonomous mobile robot navigation based on an integrated environment representation designed in dynamic environments. International Journal of Automation and Control, 11(1), 35-53.

[4] Yang, H.; Qi, J.; Miao, Y.C.; Sun, H.X.; Li, J.H. A new robot navigation algorithm based on a double-layer algorithm and trajectory optimization. IEEE Trans. Indust. Electr. 2018.

[5] Lee, H.B.; Kim, H.; Kim, H. Planning and Control for Collision-Free Cooperative Aerial Transportation. IEEE Trans. Autom. Sci. Eng. 2018, 15, 189–201.

[6] Jiang, A.; Yao, X.; Zhou, J. Research on path planning of relay obstacle avoidance of mechanical arm based on genetic algorithm. J. Eng. 2018, 16, 1579–1586.

[7] Luo, M., Hou, X., & Yang, J. (2020). Surface optimal path planning using an extended Dijkstra algorithm. IEEE access, 8, 147827-147838.

[8] Fu, B., Chen, Y., Quan, Y., Zhou, X., & Li, C. (2025). Bidirectional artificial potential field-based ant colony optimization for robot path planning. Robotics and Autonomous Systems, 183, 104834.

[9] Lv, Y.Z.; Cao, J.P. Path planning methods of mobile robot based on new neural network. In Proceedings of the 32nd Chinese Control Conference, Xi’an, China, 26–28 July 2013; pp. 3222–3226.

[10] Ma, J., Liu, Q., Yang, Z., & Wang, B. (2025). Improved Trimming Ant Colony Optimization Algorithm for Mobile Robot Path Planning. Algorithms, 18(5), 240.

[11] Dorigo, M. Optimization, Learning and Natural Algorithms. Ph.D. Thesis, Politecnico di Milano, Milano, Italy, 1992.

[12] Tao, Y., Gao, H., Ren, F., Chen, C., Wang, T., Xiong, H., & Jiang, S. (2021). A mobile service robot global path planning method based on ant colony optimization and fuzzy control. Applied Sciences, 11(8), 3605.

[13] Zhao J, Tang Y F, Jiang G P, Xu F Y, Ding J. Robot path planning based on improved ant colony optimization[J]. Journal of Nanjing University of Posts and Telecommunications (Natural Science Edition), 2019, 39(06): 73-78.

[14] Liu, Y.; Jiang, C.; Zhang, X.; Mourelatos, Z.P.; Barthlow, D.; Gorsich, D.; Singh, A.; Hu, Z. Reliability-Based Multivehicle Path Planning Under Uncertainty Using a Bio-Inspired Approach. J. Mech. Des. 2022, 144, 071701.

[15] Zhang, Z., Bai, X., Yang, H., & Zhang, S. (2026). Dynamic Path Planning via Enhanced ACO and DWA Algorithms. Applied Sciences, 16(2), 583.

[16] Li, M., Li, B., Qi, Z., Li, J., & Wu, J. (2023). Optimized APF-ACO algorithm for ship collision avoidance and path planning. Journal of Marine Science and Engineering, 11(6), 1177.

[17] Wang, H., Wang, S., & Yu, T. (2024). Path Planning of Inspection Robot Based on Improved Ant Colony Algorithm. Applied Sciences, 14(20), 9511.

[18] Tomitagawa, K., Anuntachai, A., Chotipant, S., Wongwirat, O., & Kuchii, S. (2022). Performance Measurement of Energy Optimal Path Finding for Waste Collection Robot Using ACO Algorithm. IEEE Access, 10, 117261-117272.

[19] Chen, Y., Wu, J., He, C., & Zhang, S. (2023). Intelligent warehouse robot path planning based on improved ant colony algorithm. IEEE Access, 11, 12360-12367.

[20] Wu, P., Zhong, L., Xiong, J., Zeng, Y., & Pei, M. (2023). Two-level vehicle path planning model for multi-warehouse robots with conflict solution strategies and improved ACO. Journal of Intelligent and Connected Vehicles, 6(2), 102-112.

[21] Li, Y., Cui, C., & Zhao, Q. (2025). A Comprehensive Optimization for Path Planning: Combining Improved ACO and Smoothing Techniques. Processes, 13(2), 555.

[22] Chen, Y., Bai, G., Zhan, Y., Hu, X., & Liu, J. (2021). Path planning and obstacle avoiding of the USV based on improved ACO-APF hybrid algorithm with adaptive early-warning. Ieee Access, 9, 40728-40742.

[23] Wang, L., Kan, J., Guo, J., & Wang, C. (2019). 3D path planning for the ground robot with improved ant colony optimization. Sensors, 19(4), 815.