A STUDY ON FLOOD SUSCEPTIBILITY ZONES IN ERNAKULAM DISTRICT, KERALA, INDIA USING THE FREQUENCY RATIO MODEL

Abstract:

Flooding is one of the most frequent and destructive natural hazards affecting the low-lying coastal regions of Kerala, particularly Ernakulam District, which experiences recurrent flood events during the southwest monsoon. The present study aims to delineate flood susceptibility zones in Ernakulam District, Kerala, India, using the Frequency Ratio (FR) model integrated with Geographic Information System (GIS) techniques. A flood inventory was prepared using historical flood records and satellite-based data, while six flood-conditioning factors—geology, land use/land cover (LU/LC), soil, drainage density, rainfall, and slope—were selected based on their relevance to flood occurrence. Each factor was classified and weighted using the Frequency Ratio approach to evaluate its relative contribution to flooding.

The results indicate that built-up areas exhibit the highest flood susceptibility (FR = 4.65), highlighting the impact of rapid urbanisation and increased impervious surfaces. In contrast, forest areas show minimal flood influence (FR = 0.17), emphasizing their role in runoff regulation. The factor ranking reveals that geology (FR = 7.73) is the most influential parameter, followed by LU/LC (FR = 6.90) and soil (FR = 6.58), while slope (FR = 1.60) has comparatively lower control over flood occurrence. Based on the weighted overlay of all factors, the study area was classified into different flood susceptibility zones ranging from low to very high. The findings demonstrate that the Frequency Ratio model is an effective and reliable tool for flood susceptibility assessment in data-scarce regions. The generated flood susceptibility map can support disaster risk reduction, urban planning, and sustainable flood management strategies, contributing to improved resilience against future flood events in Ernakulam District.

Keywords: Geographic Information System (GIS), Remote Sensing, Frequency ratio,

Vulnerable zones.

1. Introduction:

High concentration rainfall in a region frequently leads to flooding in the downstream areas. Floods occur after the overflow of waters inundates land (Merz et al. 2010). Natural disasters, like floods, cause huge damages to natural and human resources (Youssef et al. 2011). Flood Control and prevention are required to decrease the potential damage to natural resources, agriculture, transportation, etc. (Billa et al. 2006). Therefore, flood susceptibility is an important mission for early warning systems and emergency services for the prevention and mitigation of future floods (Tehrany et al. 2015). There are quite a few inclusive tools available and used by many research organizations worldwide. Remote sensing and GIS techniques recommend an appropriate platform to control and examine all relevant information to easily demarcate the hazard zones (Khan et al. 2008; Saha et al. 2005; Wang et al. 2013; Pourghasemi et al. 2014). RS and GIS techniques are used widely to identify and assess flood-related damages (Pradhan et al. 2014; Patel and Srivastava 2013) caused by extreme rain in a catchment area or sea wave surges in the coastal regions. One of the very common statistical-based methods is the FR model which has been run in the ArcGIS platform to prepare the Flood Susceptibility Mapping for the study area. The main perception of this method is to find out the relationship between the past flood and their causative factors which influence the occurrences of Flood in the region.

In the present study area, Ernakulam District in Kerala, severe flood events have been recorded, particularly during the 2018 extreme monsoon season, which was triggered by prolonged and intense rainfall across the district (Central Water Commission, 2018). Continuous heavy rainfall over several days led to the overflow of major rivers such as the Periyar, Muvattupuzha, and Chalakudy, resulting in widespread inundation of low-lying and coastal areas (Census Handbook 2011). Large parts of the district experienced flooding due to high rainfall intensity, reservoir releases, and poor drainage conditions, causing significant damage to life, infrastructure, and agriculture (Kerala State Biodiversity Board,2023).

2. Aim :

This study aims to delineate flood susceptibility zones in Ernakulam District using the Frequency Ratio Model to support effective flood risk management and mitigation planning.

3. Data base and Methodology:

The study was based on secondary data. The Survey of India Toposheet (SOI) at the scale of

1:50,000 is used to create a base map and Drainage map. The Indian Satellite Remote sensing

data for 1990 Landsat 5 (TM), 2017 Landsat 7 (ETM+) C1 is used to create Landuse landcover map. The SRTM images are used to create an Elevation map. Climatic data for the period of 2014-2018 is acquired from IMD. The Census of India Handbook for the year 1991, 2001 and 2011 is used for accessing the population data. The data required to flood is collected from Kerala State Disaster Management Authority (KSDMA).

The impact of the flood occurrence in the district is identified by creating Flood Vulnerability

Zones of the district by using Frequency Ratio Modelling. The Frequency Ratio Model is a

statistical-based approach that has been applied to evaluate flood susceptibility in the present

study. Also, satellite imagery, in particular, LANDSAT and ASTER DEM were the base to

create used to create the various thematic maps especially land use/land cover, Relief, Slope,

Drainage Density, Rainfall, Soil, and Geology. In addition to that, this objective implements an efficient methodology for the identification of vulnerable zone to flood hazards and the spatial extension of diseases pattern in the study area.

4. Study area:

Ernakulam district spread over an area of 3,068 square km. It covers 7.9 percent of the land area of Kerala state. The latitudinal and longitudinal extension of Ernakulam district is 9o47' and 10o18' north latitudes and 76o9' and 77o 6' east longitudes. It measures 38 km North-South and 48 km east-west.  It is bordered on the east by Idukki district, north by Thrissur district, west by Lakshadweep Sea and south by Kottayam and Alappuzha districts. The district is having a coastline of about 30km.

Figure 1 Study area

Ernakulam district is drained by River Periyar and its tributaries in the north and Muvattupuzha River in the south. This region lies on the southwestern coast of Kerala and experiences heavy rainfall from South-West Monsoon and North-East Monsoon. The district is divided into three physiographic zones as the midland region in the east, the coastal plains in the west, and the easternmost part of the area is steep to very steep hills. The coastal plain, a low lying area, with a maximum elevation of 10m towards the eastern part. This region is characterized by backwater bodies, marshy lands, sandy flats, and alluvial plains responsible for flooding during the monsoon period. The midland area has an undulating topography with low hills and narrow valleys. The hills are usually covered with laterite soils, and the valleys are alleviated. This region has a very gentle to moderate slope from east to west.

5. Results and Discussion:

5.1 Relationship between Slope and Floods:

Slope:

The slope angle is one of the main factors for the assessment of slope stability (Lee and Min, 2001). To study the Flood Susceptibility, the slope angle is very normally used so that the flood-prone area is correlated with the decreasing slope angle. It is the product of the mean slope in each pixel and is measured as a percentage (%) of the elevation change to the corresponding horizontal displacement. The slope is the main product of the elevation and gives information on the surface runoff and water concentration in a basin. Generally, the smaller the slope, the more vulnerable the flood area. In this study, the slope angle has been classified into five classes such as 0°-1°., 1°-3°, 3°-5°, 5°-7°, and 7°-9°. Table 1 shows that the majority of flood-prone areas have fallen under the 0°-1° followed by 1°-2.

Figure 3 Soil

Figure 2 Slope

(Fig 2 & 3 Source: Author's compilation)

From the Frequency Ratio table (1) shows the Frequency Ratio of Flood occurrence based on slope. In this table, the slope category of 1°-2°shows the maximum areas of 42 out of 44 which are 95.45 percent. The frequency ratio of the slope has been calculated with the percentage of flood-prone areas occurrence divided by the percentage of pixel domain which is 71.05 percent and the Frequency ratio is 1.34. Likewise, the frequency ratio for all the other slope classes has been calculated.

5.2 Soil

The soil is another important causative factor for the occurrence of the flood in the study area. Based on the rock formation of the study area, the soil has been classified into five classes such as Gravelly clay, gravelly loam, clay, loam, and sandy.

Table 1 Frequency Ratio of Flood prone areas based on Slope (Source: Author's compilation)

| Slope | Flood Occurrence | Flood occurrence % | Pixel Domain | Pixel Domain % | Frequency Ratio |
| --- | --- | --- | --- | --- | --- |
| 0-1 | 42 | 95.45 | 2117786 | 71.05291058 | 1.343429068 |
| 1 to 3 | 2 | 4.54 | 513747 | 17.2365006 | 0.263710985 |
| 3 to 5 | 0 | 0 | 220375 | 7.393705109 | 0 |
| 5 to 7 | 0 | 0 | 91125 | 3.057294966 | 0 |
| 7 to 9 | 0 | 0 | 37543 | 1.259588751 | 0 |

From the Frequency Ratio table (2) shows the Frequency Ratio of Flood prone areas based on soil types. In this table, the soil type Gravelly clay covers the maximum number of

Flood-prone areas of 15 out of 44 which is 53.1047 percent, Clay soil of 12 with 22.2702 percent, Loam soil with 10 with 10.8186 percent, and sandy soil with 7 with 6.0705 percent. The highest frequency ratio is for the sandy soil type has been calculated with the percentage of 6.0705 and the Flood occurrence divided by the percentage of pixel domain which is 2.6207. the lowest Frequency Ratio for gravelly loam soil as 0.  Likewise, the frequency ratio of all the other soil types has been calculated.

Table 2  Frequency Ratio of Flood prone areas based on Soil (Source: Author's compilation)

| Soil | Flood Occurrence | Flood occurrence % | Pixel Domain | Pixel Domain % | Frequency Ratio |
| --- | --- | --- | --- | --- | --- |
| gravelly clay | 15 | 34.09090909 | 30198 | 53.10472171 | 0.64195627 |
| gravelly loam | 0 | 0 | 4399 | 7.735865647 | 0 |
| clay | 12 | 27.27272727 | 12664 | 22.27028928 | 1.224623844 |
| loam | 10 | 22.72727273 | 6152 | 10.81860547 | 2.100758068 |
| sandy | 7 | 15.90909091 | 3452 | 6.070517893 | 2.620713947 |

5.3 Drainage Density

Another important factor is the Drainage density. Based on Natural break (Jenks) method, the drainage density layer has been classified into five classes such as 0-1 km/km2which covered 12352 pixels, 1-3 km/km2 which covered 16425, 3-5 km/km2 covered 6782, 5-7 km/km2 2559 and 7-9 km/km2 379. From the table it shows that the flood-prone areas have fallen under 3-5 km/km2 of 45.45 percent, followed by 1-3 km/km2 of 31.81 percent, 5-7 km/km2 of 15.90 percent, 0-1 km/km2 of 6.81 percent and 7-9 km/km2 of 0 percent.

Table (3) shows the Frequency Ratio of flood occurrence based on drainage density. In this table, 3-5 km/km2density class shows the areas with a maximum number of flood occurrences of 20 out of total flood 44 which is 45.45 percentage. The frequency ratio of the drainage density has been calculated with the percentage of flood occurrence divided by the percentage of pixel domain which is 2.58. Likewise, the frequency ratio of all the other density classes has been calculated.

Figure 4 Drainage Density (Source: Author's compilation)

Table 3 Table Frequency Ratio of Flood prone areas based on Drainage Density (Source: Author's compilation)

| Drainage Density | Flood Occurrence | Flood occurrence % | Pixel Domain | Pixel Domain % | Frequency Ratio |
| --- | --- | --- | --- | --- | --- |
| 0-1 | 3 | 6.818181818 | 12352 | 32.08561706 | 0.212499632 |
| 1 to 3 | 14 | 31.81818182 | 16425 | 42.66566226 | 0.745756192 |
| 3 to 5 | 20 | 45.45454545 | 6782 | 17.61695717 | 2.580158709 |
| 5 to 7 | 7 | 15.90909091 | 2559 | 6.647271216 | 2.393326584 |
| 7 to 9 | 0 | 0 | 379 | 0.984492298 | 0 |

5.4 Rainfall

Figure 5 Climate(Source: Author's compilation)

The main factor required for flood vulnerability assessment is rainfall. The enormity of flood completely depends on this factor. Based on the review of literature it was identified that the huge amount of rainfall in a very short period is an affordable condition for the existence of flood hazards. Thus the flood level can be accomplished with the amount of rainfall experienced for a particular area. In Ernakulam district, the rainfall can be experienced in both pre-monsoon and post-monsoon seasons rather higher in southwest monsoon from June to September.   The intense rainfall in this period results in the condition of the flood. The coastal region experiences heavy rainfall, which tends to flood in that region.

The Rainfall layer has been classified into five classes such as 1-2 which covered 26940 pixels, 2-3 which covered 18360, 3-5 covered 8574, 5-7 2874 and 7-9 1816. From the table, it shows that the flood-prone areas have fallen under 1-2  of 65.90 percent, followed by 3-5 of 13.63 percent, 2-3  of 11.36 percent, 7-9 of 9.09 percent and 5-7 of 0 percent.

Table 4 Table Frequency Ratio of Flood prone areas based on Rainfall (Source: Author's compilation)

| Rainfall | Flood Occurrence | Flood occurrence % | Pixel Domain | Pixel Domain % | Frequency Ratio |
| --- | --- | --- | --- | --- | --- |
| 1 to 2 | 29 | 65.90909091 | 26940 | 46.00095622 | 1.43277654 |
| 2 to 3 | 5 | 11.36363636 | 18360 | 31.3503176 | 0.362472767 |
| 3 to 5 | 6 | 13.63636364 | 8574 | 14.64039342 | 0.931420574 |
| 5 to 7 | 0 | 0 | 2874 | 4.907451677 | 0 |
| 7 to 9 | 4 | 9.090909091 | 1816 | 3.100881087 | 2.931718062 |

Table (4) shows the Frequency Ratio of flood occurrence based on rainfall. In this table, the 7-9 class shows the areas with 4 out of total 44 which is 9.09 percentage. The frequency ratio of the Rainfall class 7-9 has been calculated with the percentage of flood occurrence divided by the percentage of pixel domain which is 2.93. Likewise, the frequency ratio of all the other density classes has been calculated.

5.5 Geology

Lithology or rock type is one of the most important contributing factors to the flood. Based on the materials of the rock, the lithology of the stud area has been classified into ten categories such as Charnockite group of rocks which covered 28,963 pixels, Migmatite Complex which is covered 6,786, Sand and Silt cover 7,594, Penisular Gneissic Complex covers 9,975, Basic Rocks covers 1,800 Pegmatite/Aplite/Quartz vein covers 6, Alkaline rocks cover 10, Khondalite Group of rocks covers 183, Sandstone and clay with lignite cover 54, and Laterite with 134 pixels (table 5). The table shows that the majority of flood has occurred under Sand and silt, followed by the Charnockite group of rocks, Migmatite Complex, Peninsular Gneissic Complex, and Basic Rocks.The strong influence of geology and alluvial deposits aligns with findings by Pradhan (2010) and Saha et al. (2005), where unconsolidated sediments exhibit higher flood and hazard susceptibility.

Figure 6 Geology (Source: Author's compilation)

From the Frequency Ratio table (5) shows the Frequency Ratio of flood occurrence based on Geology. In this table, the rock type Sand and Siltshows the maximum number of landslides of 19 out of total landslides 44 which is 13.27 percent. The frequency ratio of the Geology rock type sand and silt has been calculated with the percentage of landslide occurrence divided by the percentage of pixel domain which is 3.25.  Likewise, the frequency ratio of all the other rock types has been calculated.

Table 5 Frequency Ratio of Flood prone areas based on Geology(Source: Author's compilation)

| Rock Type | Flood Occurrence | Flood occurrence % | Pixel Domain | Pixel Domain % | Frequency Ratio |
| --- | --- | --- | --- | --- | --- |
| Charnockite group of rocks | 17 | 38.63636364 | 28963 | 50.63195986 | 0.763082522 |
| Migmatite Complex | 3 | 6.818181818 | 6786 | 11.86301418 | 0.574742786 |
| Sand and Silt | 19 | 43.18181818 | 7594 | 13.27552751 | 3.252738406 |
| Penisular Gneissic Complex | 1 | 2.272727273 | 9975 | 17.43789661 | 0.13033265 |
| Basic Rocks | 1 | 2.272727273 | 1800 | 3.146688111 | 0.722260101 |
| Pegmatite/Aplite/Quartz vein | 0 | 0 | 6 | 0.01048896 | 0 |
| Alkaline Rocks | 0 | 0 | 10 | 0.017481601 | 0 |
| Khondalite Group of rocks | 0 | 0 | 183 | 0.319913291 | 0 |
| Sandstone and clay with lignite interc | 0 | 0 | 54 | 0.094400643 | 0 |
| Laterite | 0 | 0 | 131 | 0.229008968 | 0 |

5.6 Land use/Land Cover

Figure 7 Landuse/Landcover(Source: Author's compilation)

Land use /land cover changes have become the most important issue for the past 10 years in the study area. Since the last two decades, most of the forest cover area converted into the built-up land due to the pressure of urbanization and these changes leads to the occurrence of flood in the district. In the following table, the LU/LC class has been classified into five classes, such as water bodies, forest, agricultural land, Wastelands, and Built-up-land.

The high flood susceptibility of built-up areas is consistent with Tehrany et al. (2013) and Youssef et al. (2016), which report increased runoff and reduced infiltration due to impervious surfaces. In figure 7 the Land use land cover has been classified into five classes such as water bodies which covered 3,310 pixels, forest which covered 15,493, Agricultural land covered 32,707, Wasteland covered 1,177 and Built-up land covers 4,472 pixels. From the table, it shows that the flood-prone areas have fallen under agricultural land of 52.27 percent, followed by Built-up land of 36.36 percent, water bodies of 6.81 percent, Forest of 4.54 percent and wasteland of 0 percent.

Table 6 Frequency Ratio of Flood prone areas based on LU/LC(Source: Author's compilation)

| LU/LC |  | Flood Occurrence | Flood occurrence % | Pixel Domain | Pixel Domain % | Frequency Ratio |
| --- | --- | --- | --- | --- | --- | --- |
| Water bodies |  | 3 | 6.818181818 | 3310 | 5.790864081 | 1.177403186 |
| Forest |  | 2 | 4.545454545 | 15493 | 27.10509281 | 0.167697435 |
| Agricultural land |  | 23 | 52.27272727 | 32707 | 57.22108504 | 0.913522126 |
| Wastelands |  | 0 | 0 | 1177 | 2.059168285 | 0 |
| Built-up land |  | 16 | 36.36363636 | 4472 | 7.823789779 | 4.647828915 |

Table (6) shows the Frequency Ratio of flood occurrence based on LU/LC. In this table, the Built-up land shows the areas with 16 out of total 44 which is 36.36 percent. The frequency ratio of the Built-up area has been calculated with the percentage of flood occurrence divided by the percentage of pixel domain which is 4.65. Likewise, the frequency ratio of all the other density classes has been calculated.

Table 7 Rank of Flood Factors based on the Frequency Ratio Method (Source: Author's compilation)

| Rank | Factor | FR Value |
| --- | --- | --- |
| 1 | Geology | 7.73 |
| 2 | LU/LC | 6.90 |
| 3 | Soil | 6.58 |
| 4 | Drainage Density | 5.93 |
| 5 | Rainfall | 5.65 |
| 6 | Slope | 1.60 |

Flood Susceptibility Zones

Flood Susceptibility Zonation Map (FSZM) which has been prepared by the incorporation of six contributing factors by applying the FR Method on the GIS platform. All the thematic layers such as slope, geology, rainfall, land use/ land cover, soil texture, and drainage density of the study area have been converted into a raster layer with a 0.21*0.21 meter cell size. With the help of the Spatial Analyst Tool, the reclassification has been done to find out the pixel value for each layer. FR method is a kind of prediction model which is based on the calculation of flood occurred percentage and area percentage value. After getting FR value of all the raster layers, have been assigned into Weighed Sum method in Spatial Analyst Tool on the GIS platform (ArcGIS 10.3).

Thus, the ranking of each subclass of all causative factors was generated based on the FR values as shown in Table 8 FR value ranges from 0 to 7.73 in the study area. Considered FR values indicate as weak (< 1) to strong (> 1) concerning flood occurrence (Lee et al. 2012). Finally, based on the FR model in, the flood possibility database was developed. FR value in the model varied from 1.6. to 7.73. The greater value indicates a higher chance to flood occurrence. The created database was reclassified into five (5) different flood susceptibility zones, namely very low, low, moderate, high, and very high susceptibility zones (Fig 8).

Figure.8 Flood Vulnerability Map(Source: Author's compilation)

The result indicates that about 775 kilometre2 area is demarcated as very high, 843.7 kilometre2  as high zone, 929.1  kilometer2  as moderate zone, 178.31 kilometre2 as low zone covers and 178.31 kilometre2 as very low zone (table 8). Flood vulnerable zone High to very high zones is mostly located along the Western part of the study area (Fig 8).  The high to very high flood susceptibility zones are categorized with higher runoff, poorly drained soil, alluvial deposits, braided flood plain, lower slope, and lower elevation and closer to the main river, which are the significant conditioning factors for flood susceptibility mapping using the FR model.

Table 8 Flood Susceptibility Class and Flood Density (Frequency Ratio- FR Method) (Source: Author's compilation)

| Vulnerability zones | Flood Occurrence | Flood occurrence % | Pixel Domain | Pixel Domain % | area_sqkm (A) | Density (FO/A) |
| --- | --- | --- | --- | --- | --- | --- |
| Very High | 1 | 2.27 | 14413 | 25.55 | 775.76 | 0.001 |
| High | 12 | 27.27 | 15677 | 27.80 | 843.7 | 0.014 |
| Moderate | 16 | 36.36 | 17263 | 30.61 | 929.1 | 0.017 |
| Low | 10 | 22.72 | 5724 | 10.15 | 308.08 | 0.032 |
| Very Low | 5 | 11.36 | 3313 | 5.87 | 178.31 | 0.028 |
| Total of High & Very High | 13 | 29.54 | 30090 | 53.36 | 1619.46 | 0.015 |

Figure 9 Percentage of Flood with Area in each FVZ (Source: Author's compilation)

Finally, based on the study an impact analysis was done to identify the risk towards the local community in this region. About 9 Panchayaths (20.9%) are situated in the very high flood vulnerable zone and 14 Panchyaths of 51.1% in the high flood vulnerable zone. A total population of 1,37,292 is living in those vulnerable zones need special consideration from different levels of governments to take suitable actions to prevent and mitigate future flood occurrence.

6. Conclusion

This study has successfully applied the FR model to evaluate and delineate flood susceptibility zones in Ernakulam District, Kerala, using multiple flood-conditioning factors. The analysis of land use/land cover (LU/LC) clearly indicates that built-up areas exhibit the highest flood susceptibility, highlighting the strong influence of rapid urbanisation, inadequate drainage systems and impervious surfaces, on flood occurrence. Overall, the Frequency Ratio model proved to be an effective and reliable approach for identifying flood-susceptible areas in the study region. To reduce future flood risk, the findings highlight the urgent need for sustainable land-use planning, floodplain management, controlled urban expansion, and improved drainage infrastructure. The flood susceptibility map created can serve as a valuable decision-support tool for planners, disaster management authorities, and policymakers in implementing targeted flood mitigation and climate-resilient development strategies in Ernakulam District.

Disclaimer (Artificial intelligence)

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

References:

- Billa, L., Mansor, S., & Mahmud, A.R. (2004). Spatial information technology in flood early warning systems: An overview of theory, application and latest developments in Malaysia. Disaster Prevention and Management, 13(5), 356-363.

- Central Water Commission. (2018). Study report on Kerala floods of August 2018.

- Kerala State Biodiversity Board. (2023). Impact of the 2018 flood in Kerala.

- Khan, F., Kanwal, S., Alamri, S., & Mumtaz, B. (2020). Hyperparameter optimization of classifiers, using an artificial immune network and its application to software bug prediction. IEEE Access, 8, 20954–20964.

- Merz, B., Kreibich, H., Schwarze, R., and Thieken, A.: Review article "Assessment of economic flood damage", Nat. Hazards Earth Syst. Sci., 10, 1697–1724, https://doi.org/10.5194/nhess-10-1697-2010, 2010.

- Pradhan, B. (2010). Landslide susceptibility mapping of a catchment area using frequency ratio, fuzzy logic and multivariate logistic regression approaches. Journal of the Indian Society of Remote Sensing, 38(2), 301–320.

- Saaty, R.W. 1987. “The Analytic Hierarchy Process—what it is and how it is used.” Mathl Modelling 9, no. 3–5: 161–176.

- Saha, A.K., R.P. Gupta, I. Sarkar, M.K. Arora, and E. Csaplovics. 2005. “An Approach for GISBased Statistical Landslide Susceptibility Zonation—with a Case Study in the Himalayas.” Landslides 2: 61–69. https://doi.org/10.1007/s10346-004-0039-8.

- Tan, Q., M. Bai, P. Zhou, J. Hu, and X. Qin. 2020. “Geological Hazard Risk Assessment of Line Landslide Based on Remotely Sensed Data and GIS.” Measurement 169: 10. https://doi.org/10.1016/j.measurement.2020.108370.

- Tehrany, M. S., Pradhan, B., & Jebur, M. N. (2013). Spatial prediction of flood susceptible areas using rule based decision tree (dt) and a novel ensemble bivariate and multivariate statistical models in Gis. Journal of Hydrology, 504, 69–79.

- Wang, G. and K. Sassa. 2009. “Seismic Loading Impacts on Excess Pore-Water Pressure Maintain Landslide Triggered Flowslides.” Earth Surface Processes and Landforms 34: 232–241. https://doi.org/10.1002/esp.1708.

- Youssef, A.M., et al. (2016) Analysis on Causes of Flash Flood in Jeddah City (Kingdom of Saudi Arabia) of 2009 and 2011 using Multi-Sensor Remote Sensing Data and GIS. Geomatics, Natural Hazards and Risk, 7, 1018-1042.
https://doi.org/10.1080/19475705.2015.1012750