Review Article

Nanobubble & Plasma Water Technology in Aquaculture

Abstract

Technologies such as nanobubble and plasma water are evolving to be potentially effective sustainable aquaculture solutions to boost productivity, health and sustainability. The unique characteristics such as prolonged residence time, high gas transfer efficiency and the production of reactive oxygen species make nanobubbles an efficient agent in enhancing water quality, increasing dissolved oxygen content and controlling pathogenic microorganisms. Plasma activated water (PAW) is produced by exposing water to non-thermal plasma and is a complex mixture of reactive species that has been successfully used in the disinfection of waters, promotion of fish growth and immunity. This review provides an overview of the principles, generation methods, and possible applications of nanobubble and plasma water technologies in aquaculture. This perspective evaluates the roles of these technologies to improve aquaculture efficiency, their opportunities and challenges and future research directions needed for this important technology advancement to help transition the aquaculture industry towards more sustainable and environmentally friendly practices.

Keywords: Aquaculture; Fish Health; Nanobubbles; Plasma-Activated Water; Water Quality

Introduction

Aquaculture, the farming of aquatic organisms in controlled and semi-controlled environments, plays a crucial role in meeting the world's growing demand for protein. But these booming aquaculture systems have come at a price: declining water quality and disease have become increasingly common problems. These new technologies are no longer experimental and the best way forward for the sustainable development of aquaculture. The use of nanobubbles and plasma activation of water in aquaculture systems can significantly improve water quality, fish health, and productivity.

Nanobubbles are bubbles with diameter less than 200 nm. Nanobubbles have unique properties, including high stability, extended residence time, and a large surface-area-to-volume ratio. Such features enable nanobubbles to act with high efficacy in gas solubilisation, reactive oxygen species production, and biology interaction. The use of nanobubbles for environmental management such as aquaculture, which offers applications in water quality parameters (e.g., dissolved oxygen) and the degradation of toxic compounds such as ammonia and nitrite has also been explored with studies indicating that nanobubble aeration can significantly reduce total ammonia nitrogen levels in shrimp farming environments (Susanti et al., 2021)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkxpbHkiLCJmYW1pbHkiOiJTdXNhbnRpIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlN1eXVkIFdhcm5vIiwiZmFtaWx5IjoiVXRvbW8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTm92ZXJpdGEgRGlhbiIsImZhbWlseSI6IlRha2FyaW5hIn19XSwiaWRzIjp7Imlzc24iOlsiMjExNy00NDU4IiwiMjI3My0xNzA5Il0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDUxL2Jpb2NvbmYvMjAyMTMzMDUwMDUiLCJtYWciOiIzMTkzNTYzNzAzIiwib3BlbmFsZXgiOiJXMzE5MzU2MzcwMyIsIm9wZW5BY2Nlc3NJZCI6IjU2NTM0MzU3NjIzODY0OTcxMzcifSwidGl0bGUiOiJTdXN0YWluYWJpbGl0eSBhbmQgZmVhc2liaWxpdHkgYXNzZXNzbWVudHMgb2YgbmFub2J1YmJsZSBhZXJhdGlvbiB0ZWNobm9sb2d5IGluIGVjb25vbWljLXNvY2lvIGVudmlyb25tZW50IG9mIFBlbmFldXMgdmFubmFtZWkgc2hyaW1wIGZhcm1pbmciLCJhYnN0cmFjdCI6IlBlbmFldXMgdmFubmFtZWkgc2hyaW1wIGZhcm1pbmcgaW4gSW5kb25lc2lhIGlzIGZhY2VkIHdpdGggc2V2ZXJhbCBjaGFsbGVuZ2VzIGluIHRoZSBlbnZpcm9ubWVudGFsLCBlY29ub21pYywgYW5kIHNvY2lhbCBhc3BlY3RzLiBUaGVyZWZvcmUsIHRoaXMgc3R1ZHkgYWltcyB0byBhc3Nlc3MgdGhlIGJlbmVmaXRzIG9mIG5vdmVsIG5hbm9idWJibGUgYWVyYXRpb24gc3lzdGVtcyBmb3IgdGhlIGVjb3N5c3RlbSwgYnVzaW5lc3NlcywgYW5kIGNvbW11bml0aWVzLiBUaGlzIGlzIGFuIGV4cGVyaW1lbnRhbCBzdHVkeSBjb25kdWN0ZWQgYnkgY29tcGFyaW5nIFAuIHZhbm5hbWVpIHBvc3QgbGFydmEgMTAgcmVhcmVkIGF0IGEgZGVuc2l0eSBvZiAyMDAwIGluZHMuL0wgaW4gYSBwb25kIHRyZWF0ZWQgd2l0aCBuYW5vYnViYmxlIGFuZCB3aXRob3V0IHRyZWF0bWVudHMgKGNvbnRyb2wpLiBGdXJ0aGVybW9yZSwgdGhlIE5ldCBQcmVzZW50IFZhbHVlIChOUFYpIGFuZCBwYXliYWNrIHBlcmlvZCB3ZXJlIGNhbGN1bGF0ZWQgdG8gYXNzZXNzIHRoZSBlY29ub21pYyBmZWFzaWJpbGl0eSBvZiBuYW5vYnViYmxlLCB3aGlsZSBjb21tdW5pdHkgaW50ZXJ2aWV3cyB3ZXJlIHVzZWQgZm9yIHRoZSBzb2NpYWwgc2VjdG9ycy4gQmFzZWQgb24gdGhlIHJlc3VsdHMsIGVudmlyb25tZW50YWwgd2FzdGVzIG9mIHNocmltcCBmYXJtaW5nIGluIGZvcm0gb2YgdG90YWwgYW1tb25pYSBuaXRyb2dlbiAoVEFOKSB3ZXJlIHJlZHVjZWQgYnkgOSUgZnJvbSAyLjU4IG1nL2wgKDk1JUNJOiAwLDkxLCA0LDI1KSBpbiBjb250cm9sIGNvbXBhcmVkIHRvIDIuMzUgbWcvbCAoOTUlQ0k6IDAsODYsIDMsODQpIGluIHRyZWF0bWVudC4gRnVydGhlcm1vcmUsIHRoZSByZXZlbnVlcyBmcm9tIHBvc3QgbGFydmEgc2FsZXMgZm9yIGZpdmUgeWVhcnMgYW5kIG5hbm9idWJibGUgaW52ZXN0bWVudCBjb3N0cyBzaG93ZWQgdGhhdCB0aGUgZXN0aW1hdGVkIE5QViB3YXMgSURSIDY0LDgyNCwzNzQgd2l0aCBhIHBheWJhY2sgcGVyaW9kIG9mIDEuNyB5ZWFycy4gVGhlIGludGVydmlld3Mgb24gdHJhZGl0aW9uYWwgc2hyaW1wIGZhcm1pbmcgc2hvd2VkIHRoYXQgNjEuMSUgdG8gNzIuMiUgb2YgY29tbXVuaXR5IG1lbWJlcnMgYWdyZWVkIG9uIHRoZSB1c2Ugb2YgbmFub2J1YmJsZSBhZXJhdGlvbiB0byBzdXBwb3J0IGxpdmVsaWhvb2RzLiBUaGVyZWZvcmUsIHRoZSB1c2Ugb2YgbmFub2J1YmJsZXMgaXMgZmVhc2libGUgdG8gc3VwcG9ydCBzdXN0YWluYWJsZSBQLiB2YW5uYW1laSBmYXJtaW5nLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDUxL2Jpb2NvbmYvMjAyMTMzMDUwMDUiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vd3d3LmJpby1jb25mZXJlbmNlcy5vcmcvYXJ0aWNsZXMvYmlvY29uZi9wZGYvMjAyMS8wNS9iaW9jb25mX2ljYXZlc3MyMDIxXzA1MDA1LnBkZiIsImlzc3VlZERhdGUiOiIyMDIxLTAxLTAxIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0wOSIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiQklPIFdlYiBvZiBDb25mZXJlbmNlcyIsInZvbHVtZSI6MzMsImltcGFjdEZhY3RvciI6IjAuNjIifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjo1MDA1LCJsYXN0UGFnZSI6NTAwNX19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJFRFAgU2NpZW5jZXMifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjExLCJjaXRlZEJ5Q291bnQiOjR9fSwicXVvdGUiOiJBYnN0cmFjdC4gUGVuYWV1cyB2YW5uYW1laSBzaHJpbXAgZmFybWluZyBpbiBJbmRvbmVzaWEgaXMgZmFjZWQgd2l0aCBzZXZlcmFsIGNoYWxsZW5nZXMgaW4gdGhlIGVudmlyb25tZW50YWwsIGVjb25vbWljLCBhbmQgc29jaWFsIGFzcGVjdHMuICBUaGVyZWZvcmUsIHRoaXMgc3R1ZHkgYWltcyB0byBhc3Nlc3MgdGhlIGJlbmVmaXRzIG9mIG5vdmVsIG5hbm9idWJibGUgIGFlcmF0aW9uIHN5c3RlbXMgZm9yIHRoZSBlY29zeXN0ZW0sIGJ1c2luZXNzZXMsIGFuZCBjb21tdW5pdGllcy4gVGhpcyBpcyBhbiAgZXhwZXJpbWVudGFsIHN0dWR5IGNvbmR1Y3RlZCBieSBjb21wYXJpbmcgUC4gdmFubmFtZWkgcG9zdCBsYXJ2YSAxMCAgcmVhcmVkIGF0IGEgZGVuc2l0eSBvZiAyMDAwIGluZHMuL0wgaW4gYSBwb25kIHRyZWF0ZWQgd2l0aCBuYW5vYnViYmxlIGFuZCAgd2l0aG91dCB0cmVhdG1lbnRzIChjb250cm9sKS4gRnVydGhlcm1vcmUsIHRoZSBOZXQgUHJlc2VudCBWYWx1ZSAoTlBWKSBhbmQgIHBheWJhY2sgcGVyaW9kIHdlcmUgY2FsY3VsYXRlZCB0byBhc3Nlc3MgdGhlIGVjb25vbWljIGZlYXNpYmlsaXR5IG9mICBuYW5vYnViYmxlLCB3aGlsZSBjb21tdW5pdHkgaW50ZXJ2aWV3cyB3ZXJlIHVzZWQgZm9yIHRoZSBzb2NpYWwgc2VjdG9ycy4gIEJhc2VkIG9uIHRoZSByZXN1bHRzLCBlbnZpcm9ubWVudGFsIHdhc3RlcyBvZiBzaHJpbXAgZmFybWluZyBpbiBmb3JtIG9mICB0b3RhbCBhbW1vbmlhIG5pdHJvZ2VuIChUQU4pIHdlcmUgcmVkdWNlZCBieSA5JSBmcm9tIDIuNTggbWcvbCAoOTUlQ0k6ICAwLDkxLCA0LDI1KSBpbiBjb250cm9sIGNvbXBhcmVkIHRvIDIuMzUgbWcvbCAoOTUlQ0k6IDAsODYsIDMsODQpIGluICB0cmVhdG1lbnQuIEZ1cnRoZXJtb3JlLCB0aGUgcmV2ZW51ZXMgZnJvbSBwb3N0IGxhcnZhIHNhbGVzIGZvciBmaXZlIHllYXJzIGFuZCAgbmFub2J1YmJsZSBpbnZlc3RtZW50IGNvc3RzIHNob3dlZCB0aGF0IHRoZSBlc3RpbWF0ZWQgTlBWIHdhcyBJRFIgIDY0LDgyNCwzNzQgd2l0aCBhIHBheWJhY2sgcGVyaW9kIG9mIDEuNyB5ZWFycy4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo0NS40ODMyODc4MTEyNzkzLCJsZWZ0IjoyMS44Mjc3NzAyMzMxNTQyOTcsIndpZHRoIjo1Ni44MDA1Njc2MjY5NTMxMjUsImhlaWdodCI6MS43NDkyMzg2MTAyNjc2MzkyfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NDcuMTY3MTcxNDc4MjcxNDg0LCJsZWZ0IjoyMS44Mjc3NzAyMzMxNTQyOTcsIndpZHRoIjo1Ni43NzUwMzk2NzI4NTE1NiwiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo0OC42MjM4MDk4MTQ0NTMxMjUsImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2Ljc3MTMwMTI2OTUzMTI1LCJoZWlnaHQiOjEuNDA2Nzg1OTY0OTY1ODIwM319LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjUwLjA4MDQ0ODE1MDYzNDc2NiwibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuODIwMjk3MjQxMjEwOTQsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NTEuNTM3MDkwMzAxNTEzNjcsImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2LjgyNjUzMDQ1NjU0Mjk3LCJoZWlnaHQiOjEuNDA2Nzg1OTY0OTY1ODIwM319LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjUzLjAxMDY2NTg5MzU1NDY5LCJsZWZ0IjoyMS44Mjc3NzAyMzMxNTQyOTcsIndpZHRoIjo1Ni43NjU3MjQxODIxMjg5MDYsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NTQuNDY3MzA0MjI5NzM2MzMsImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2Ljc5MzcxMjYxNTk2NjgsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NTUuOTIzOTQ2MzgwNjE1MjM0LCJsZWZ0IjoyMS44Mjc3NzAyMzMxNTQyOTcsIndpZHRoIjo1Ni43OTU2MTYxNDk5MDIzNDQsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NTcuMzgwNTg4NTMxNDk0MTQsImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2Ljc3NjkyNzk0Nzk5ODA1LCJoZWlnaHQiOjEuNDA2Nzg1OTY0OTY1ODIwM319LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjU4Ljg1NDE2NDEyMzUzNTE1NiwibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuNzgwNjY2MzUxMzE4MzYsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NjAuMzEwNzk4NjQ1MDE5NTMsImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2LjgzMzM3NDAyMzQzNzUsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NjEuNzY3NDQwNzk1ODk4NDQsImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2Ljc0NzAwOTI3NzM0Mzc1LCJoZWlnaHQiOjEuNDA2Nzg1OTY0OTY1ODIwM319LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjYzLjIyODMxMzQ0NjA0NDkyLCJsZWZ0IjoyMS44Mjc3NzAyMzMxNTQyOTcsIndpZHRoIjo1Ni43ODgxMjc4OTkxNjk5MiwiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2NC43MDE4ODkwMzgwODU5NCwibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuNzg2MjYyNTEyMjA3MDMsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NjYuMTU4NTMxMTg4OTY0ODQsImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2LjgxNTMxOTA2MTI3OTMsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX1dfSwiY2h1bmtJZCI6ImEwY2Q0YTU1LTEyMGUtNGM4My1iZjkxLTc5NWQ3OGZlYTcxMiIsImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjUwMDUifX1d. Furthermore, the high internal pressure and stability of these micro-scale gas pockets facilitate the efficient oxidation of organic matter and the suppression of pathogenic bacteria, thereby reducing reliance on chemical therapeutics (Nghĩa et al., 2022)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5ndXnhu4VuIEjhu691IiwiZmFtaWx5IjoiTmdoxKlhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5ndXllbiBUaGkiLCJmYW1pbHkiOiJOZ3V5ZW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUGhhbiBUcuG7jW5nIiwiZmFtaWx5IjoiQsOsbmgifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTGUgVGhpIiwiZmFtaWx5IjoiTWF5In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRvbmcgVHJhbiIsImZhbWlseSI6Ikh1eSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQaOG6oW0gVGjDoWkiLCJmYW1pbHkiOiJHaWFuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTb3BoaWUiLCJmYW1pbHkiOiJTdOKAkEhpbGFpcmUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUGhhbiBUaGkiLCJmYW1pbHkiOiJWYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMjM0LTE3NDkiLCIyMjM0LTE3NTciXSwiaXNibiI6W10sImRvaSI6IjEwLjQ3ODUzL2Zhcy4yMDIyLmUzOSIsIm9wZW5hbGV4IjoiVzQyOTQ1OTc1NTQiLCJvcGVuQWNjZXNzSWQiOiIxMzQ4NTgzNDA1NjEzOTM2NDAzIn0sInRpdGxlIjoiRWZmZWN0IG9mIG5hbm9idWJibGVzIChveHlnZW4sIG96b25lKSBvbiB0aGUgUGFjaWZpYyB3aGl0ZSBzaHJpbXAgKFBlbmFldXMgdmFubmFtZWkpLCBWaWJyaW8gcGFyYWhhZW1vbHl0aWN1cyBhbmQgd2F0ZXIgcXVhbGl0eSB1bmRlciBsYWIgY29uZGl0aW9ucyIsImFic3RyYWN0IjoiVGhpcyBzdHVkeSBhc3Nlc3NlZCB0aGUgZWZmZWN0cyBvZiBveHlnZW4gYW5kIG96b25lIG5hbm9idWJibGVzIG9uIGdpbGwgbW9ycGhvbG9neSwgd2VpZ2h0IGdhaW4sIGFuZCBtb3J0YWxpdHkgb2YgUGFjaWZpYyB3aGl0ZSBzaHJpbXAgKFBlbmFldXMgdmFubmFtZWkpLCBhcyB3ZWxsIGFzIHRoZSBsZXZlbCBvZiBWaWJyaW8gcGFyYWhhZW1vbHl0aWN1cyBhbmQgd2F0ZXIgcXVhbGl0eSBvZiBzaHJpbXAgY3VsdHVyZSB0YW5rcyB1bmRlciBsYWIgY29uZGl0aW9ucy4gVHdvIGV4cGVyaW1lbnRzIHdlcmUgY2FycmllZCBvdXQgd2l0aCBveHlnZW4gbWFjcm9idWJibGUsIG96b25lIG1hY3JvYnViYmxlLCBveHlnZW4gbmFub2J1YmJsZSwgb3pvbmUgbmFub2J1YmJsZSwgYW5kIGNvbnRyb2wgdHJlYXRtZW50cyAoYWlyLXN0b25lIG1hY3JvYnViYmxlKS4gRXhwZXJpbWVudHMgd2VyZSBkb25lIGluIHRyaXBsaWNhdGUgaW4gMTAwIEwgdGFua3Mgd2l0aCAxNeKAsCBzYWxpbmUgd2F0ZXIsIGFuZCAyMCBzaHJpbXAgcGVyIHRhbmsuIFRhbmtzIGluIEV4cGVyaW1lbnQgMSB3ZXJlIG5vdCBpbm9jdWxhdGVkIHdpdGggYmFjdGVyaWE7IHRhbmtzIGluIEV4cGVyaW1lbnQgMiB3ZXJlIGlub2N1bGF0ZWQgd2l0aCBWLiBwYXJhaGFlbW9seXRpY3VzIGF0IGEgY29uY2VudHJhdGlvbiBvZiAxMDYgQ0ZVL21MLiBUaGUgcmVzdWx0cyByZXZlYWxlZCB0aGF0IHNob3J0IHRyZWF0bWVudHMgd2l0aCBvem9uZSBuYW5vYnViYmxlcyBoYWQgbWluaW1hbCBpbXBhY3Qgb24gc2hyaW1wIGdpbGxzLCBtb3J0YWxpdHksIGFuZCBncm93dGggcmF0ZXMsIHJlZHVjZWQgVi4gcGFyYWhhZW1vbHl0aWN1cyBjb25jZW50cmF0aW9uIGluIHdhdGVyIGNvbXBhcmVkIHRvIHRoZSBvdGhlciBncm91cHMsIGFuZCBpbXByb3ZlIHdhdGVyIHF1YWxpdHkuIFRoZXNlIGxhYm9yYXRvcnkgcmVzdWx0cyBpbmRpY2F0ZSB0aGF0IG96b25lIG5hbm9idWJibGUgdHJlYXRtZW50IG1heSBiZSB1c2VmdWwgZm9yIGNvbnRyb2xsaW5nIFYuIHBhcmFoYWVtb2x5dGljdXMuIE1vcmUgd29yayBpcyBuZWVkZWQgdG8gZmluZCB0aGUgYmVzdCBwcm90b2NvbCB0byBhcHBseSB0aGUgdGVjaG5vbG9neSBvbiBhIGNvbW1lcmNpYWwgc2NhbGUuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjQ3ODUzL2Zhcy4yMDIyLmUzOSIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cuZS1mYXMub3JnL2Rvd25sb2FkL2Rvd25sb2FkX3BkZj9waWQ9ZmFzLTI1LTgtNDI5IiwiaXNzdWVkRGF0ZSI6IjIwMjItMDgtMDEiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA5IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJGaXNoZXJpZXMgYW5kIGFxdWF0aWMgc2NpZW5jZXMiLCJpc3N1ZSI6OCwidm9sdW1lIjoyNSwiaW1wYWN0RmFjdG9yIjoiMS4zNCJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjQyOSwibGFzdFBhZ2UiOjQ0MH19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJTcHJpbmdlciBTY2llbmNlK0J1c2luZXNzIE1lZGlhIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjozMCwiY2l0ZWRCeUNvdW50IjoxM319LCJxdW90ZSI6IlRoZSBpbmR1c3RyeSB0aGVyZWZvcmUgbmVlZHMgdG8gZmluZCBhbHRlcm5hdGl2ZSBtZWFzdXJlcyAgdG8gcHJldmVudCBhbmQgY29udHJvbCB0aGlzIHBhdGhvZ2VuLiBOYW5vYnViYmxlIHRlY2hub2xvZ3kgcHJvZHVjZXMgc21hbGwgZ2FzIGJ1YmJsZXMgd2l0aCBhIGRpYW1ldGVyIG9mIGxlc3MgdGhhbiAyMDAgbm0gKEFncmF3YWwgZXQgYWwuLCAyMDExOyBDaGFwbGluLCAyMDIxKSB3aGljaCBjYW4gZXhpc3QgaW4gdGhlIHdhdGVyIGNvbHVtbiBmb3Igd2Vla3MgKEF6ZXZlZG8gIGV0IGFsLiwgMjAxNjsgUGFybWFyICYgTWFqdW1kZXIsIDIwMTMpLiBUaGlzIHBoZW5vbWVub24gaXMgIGFzc29jaWF0ZWQgd2l0aCB0aGUgcGh5c2ljYWwgY2hhcmFjdGVyaXN0aWMgb2YgdGhlc2Ugc21hbGwgYnViLWJsZXMgKFdlaWpzICYgTG9oc2UsIDIwMTMpIGFuZCBoaWdoIHpldGEgcG90ZW50aWFsIHZhbHVlIHdoaWNoICBrZWVwcyB0aGVtIGZyb20gYWdncmVnYXRpbmcgaW50byBsYXJnZXIgYnViYmxlcyAoS2lyYnksIDIwMTA7ICBTam9ncmVlbiBldCBhbC4sIDIwMTgpLiBOYW5vYnViYmxlIHRlY2hub2xvZ3kgaGFzIGEgbG90IG9mIHByb20taXNpbmcgZWZmZWN0aXZlIGFwcGxpY2F0aW9uIGluIGFxdWFjdWx0dXJlIGJlY2F1c2Ugb2YgaXRzIGFiaWxpdHkgIHRvIGluY3JlYXNlIGFuZCBtYWludGFpbiBveHlnZW4gY29udGVudCBpbiB3YXRlciBhbmQgY29udHJvbCAgYmFjdGVyaWEsIGEgcG9zc2libGUgbWVhc3VyZSB0byByZWR1Y2UgdGhlIHVzZSBvZiBhbnRpYmlvdGljcy4gT3h5Z2VuIG5hbm9idWJibGVzIG9wdGltaXplIHRoZSBkaXNzb2x2ZWQgb3h5Z2VuIGluIGFxdWF0aWMgc3lzdGVtcyAoR2FsYW5nIGV0IGFsLiwgMjAxOSkuIEhpZ2ggbGV2ZWxzIG9mIG94eWdlbiBtYXkgc3BlZWQgdXAgbWV0YWJvbGlzbSwgcHJvbW90ZSBoaWdoZXIgZm9vZCBpbnRha2UsIGFuZCBpbmNyZWFzZSAgZ3Jvd3RoIG9mIGZhcm1lZCBzaHJpbXAgKEViaW5hIGV0IGFsLiwgMjAxMykuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzUuMDY4NjQ1NDc3Mjk0OTIsImxlZnQiOjguMDk1MjM1ODI0NTg0OTYxLCJ3aWR0aCI6NDEuMDcyMjEyMjE5MjM4MjgsImhlaWdodCI6MS42OTk2MzUxNDgwNDg0MDA5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzYuODMyNTMwOTc1MzQxOCwibGVmdCI6OC4wOTUyMzU4MjQ1ODQ5NjEsIndpZHRoIjoyMy42MDQxODUxMDQzNzAxMTcsImhlaWdodCI6MS42OTk2MzUxNDgwNDg0MDA5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzguNTk2NDIwMjg4MDg1OTQsImxlZnQiOjEwLjk1MjM2NDkyMTU2OTgyNCwid2lkdGgiOjM4LjIxNTg3NzUzMjk1ODk4NCwiaGVpZ2h0IjoxLjY5OTYzNTE0ODA0ODQwMDl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0MC4zNjAzMDk2MDA4MzAwOCwibGVmdCI6OC4wOTUyMzU4MjQ1ODQ5NjEsIndpZHRoIjo0MS4wNzY3NTkzMzgzNzg5MDYsImhlaWdodCI6MTcuNTc0NjMwNzM3MzA0Njg4fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTcuOTk5MTk1MDk4ODc2OTUsImxlZnQiOjEwLjk1MjM2NDkyMTU2OTgyNCwid2lkdGgiOjM4LjIwMTQ0NjUzMzIwMzEyNSwiaGVpZ2h0IjoxLjY5OTYzNTE0ODA0ODQwMDl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1OS43NjMwODA1OTY5MjM4MywibGVmdCI6OC4wOTUyMzU4MjQ1ODQ5NjEsIndpZHRoIjo0MS4wNjMzMjc3ODkzMDY2NCwiaGVpZ2h0IjoxLjY5OTYzNTE0ODA0ODQwMDl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2MS41MjY5NzM3MjQzNjUyMzQsImxlZnQiOjguMDk1MjM1ODI0NTg0OTYxLCJ3aWR0aCI6NDEuMDY2ODQ0OTQwMTg1NTUsImhlaWdodCI6MS42OTk2MzUxNDgwNDg0MDA5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NjMuMjkwODYzMDM3MTA5Mzc1LCJsZWZ0Ijo4LjA5NTIzNTgyNDU4NDk2MSwid2lkdGgiOjQwLjcxNDI5MDYxODg5NjQ4NCwiaGVpZ2h0IjoxLjY5OTYzMTMzMzM1MTEzNTN9fV19LCJjaHVua0lkIjoiYzViOTI3OWMtNzg0ZS00MWQ0LTg3ZjQtZjg2MmQ2OGZmMGZhIiwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNDMwIn19XQ%3D%3D.

Plasma-activated water (PAW) is a solution created by saturating cold distilled water with non-thermal plasma, which induces the generation of reactive species, including hydrogen peroxide, nitrites and reactive oxygen and nitrogen species. PAW possesses strong antimicrobial properties and plant growth-stimulating activity. In the aquaculture field, PAW is used as a water disinfectant to control pathogen disease and promote fish growth. Specifically, the reactive oxygen and nitrogen species generated during plasma activation can effectively inactivate harmful microorganisms such as Vibrio parahaemolyticus, thereby enhancing biosecurity without altering the chemical composition of the cultured product (Vichiansan et al., 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5vcnJhcG9uIiwiZmFtaWx5IjoiVmljaGlhbnNhbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQb3Jua2FyYW4iLCJmYW1pbHkiOiJQaGFuLXVtb25nIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRoaXBwYXdhbiIsImZhbWlseSI6Ik1la3dpbGFpIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkphdHVyb25nIiwiZmFtaWx5IjoiS3VtbGEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiS29tZ3JpdCIsImZhbWlseSI6Ikxla3Nha3VsIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNhaXNhbW9ybiIsImZhbWlseSI6Ikx1bXlvbmcifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiRGhlZXJhd2FuIiwiZmFtaWx5IjoiQm9vbnlhd2FuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlBpcGF0aCIsImZhbWlseSI6IlBvcmFtYXBpaml0d2F0In19XSwiaWRzIjp7Imlzc24iOltdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMjEyMDMvcnMuMy5ycy0zMjMwNTYxL3YxIiwib3BlbmFsZXgiOiJXNDM4NTk1NDk1MyIsIm9wZW5BY2Nlc3NJZCI6IjE2MTA0MDEzOTgzMzYwMzE3NzA2In0sInRpdGxlIjoiQXBwcm9wcmlhdGUgUGFyYW1ldGVycyBmb3IgZ3Jvd3RoIFJlZHVjdGlvbiBvZiBtaWNyb29yZ2FuaXNtcyBpbiBXaGl0ZSBsZWcgU2hyaW1wIChMaXRvcGVuYWV1cyB2YW5uYW1laSkgYW5kIFNwbGVuZGlkIFNxdWlkIChMb2xpZ28gZm9ybW9zYW5hKSBieSBQbGFzbWEgQWN0aXZhdGVkIFdhdGVyIiwiYWJzdHJhY3QiOiJBYnN0cmFjdCBUaGUgc3R1ZHkgZm9jdXNlcyBvbiB0aGUgYXBwbGljYXRpb24gb2YgUEFXIGluYWN0aXZhdGlvbiBmb3IgZXhwb3J0aW5nIHdoaXRlIGxlZyBzaHJpbXAgYW5kIHNwbGVuZGlkIHNxdWlkLiBUaGUgZXhwZXJpbWVudCBkZXRlcm1pbmVkIHRoZSBtYXhpbXVtIGh5ZHJvZ2VuIHBlcm94aWRlIGNvbmNlbnRyYXRpb24gaW4gUEFXIGJ5IHV0aWxpemluZyBhIGZ1bGwgZmFjdG9yaWFsIGRlc2lnbiB3aXRoIHR3by1sZXZlbCBmYWN0b3JzLiBTdWJzZXF1ZW50bHksIHRoZSBhbmFseXNpcyBvZiB2YXJpYW5jZSAoQU5PVkEpIHdhcyBjb25kdWN0ZWQgdXNpbmcgTWluaXRhYi4gVGhlIHJlc3VsdHMgaW5kaWNhdGVkIHRoYXQgYSBnYXMgZmxvdyByYXRlIG9mIGEgbWl4dHVyZSBvZiBBciAoOTglKSBhbmQgTyAyICgyJSkgYXQgMjUgTC9taW4sIHdpdGggYSBkaXNjaGFyZ2UgdGltZSBvZiAxMjAgbWludXRlcywgZ2VuZXJhdGVkIHRoZSBvcHRpbWFsIGNvbmNlbnRyYXRpb24gb2YgSCAyIE8gMiBpbiBQQVcgYXQgMTMuMzAgbWcvTCwgYXMgcGVyIHRoZSByZXNwb25zZSBvcHRpbWl6ZXIuIFRoZSB1c2Ugb2YgUEFXIHRvIGluaGliaXQgdGhlIGdyb3d0aCBvZiBtaWNyb29yZ2FuaXNtcyBpbiB3aGl0ZSBsZWcgc2hyaW1wIGFuZCBzcGxlbmRpZCBzcXVpZCBkZW1vbnN0cmF0ZWQgdGhhdCBpbmFjdGl2YXRpb24gd2l0aCBQQVcsIHNvYWtlZCBmb3IgMTAgbWludXRlcywgcmVzdWx0ZWQgaW4gYSByZWR1Y3Rpb24gb2YgdG90YWwgbWljcm9vcmdhbmlzbXMgdG8gbGVzcyB0aGFuIDYgTG9nIENGVS9nLiBBZGRpdGlvbmFsbHksIGFuIGV4YW1pbmF0aW9uIG9mIHRoZSBudXRyaXRpb24gb2YgYXF1YWN1bHR1cmUgZm9vZCByZXZlYWxlZCB0aGF0IFBBVy1pbW1lcnNlZCB3aGl0ZSBsZWcgc2hyaW1wIGFuZCBzcGxlbmRpZCBzcXVpZCBzaG93ZWQgZGlmZmVyZW50IGVmZmVjdHMgb24gcHJvdGVpbiBhbmQgbGlwaWQgY2hhbmdlcy4gVGhlIHJlc3VsdHMgc3VnZ2VzdCB0aGF0IFBBVyBjb3VsZCBiZSBhIHZpYWJsZSBvcHRpb24gZm9yIGRlY29udGFtaW5hdGluZyBhcXVhdGljIGZvb2QgcHJvZHVjdHMuIEhvd2V2ZXIsIGl0IGlzIGltcG9ydGFudCB0byBjb25zaWRlciB0aGF0IG94aWRhdGlvbiBtYXkgcG9zZSBhIGNoYWxsZW5nZSBpbiBjZXJ0YWluIHByb2R1Y3RzLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4yMTIwMy9ycy4zLnJzLTMyMzA1NjEvdjEiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vd3d3LnJlc2VhcmNoc3F1YXJlLmNvbS9hcnRpY2xlL3JzLTMyMzA1NjEvbGF0ZXN0LnBkZiIsImlzc3VlZERhdGUiOiIyMDIzLTA4LTE3IiwiYWNjZXNzZWREYXRlIjoiMjAyNS0wOCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiUmVzZWFyY2ggU3F1YXJlIChSZXNlYXJjaCBTcXVhcmUpIiwiaW1wYWN0RmFjdG9yIjoiMC40MCJ9LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJSZXNlYXJjaCBTcXVhcmUgKFVuaXRlZCBTdGF0ZXMpIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo0MCwiY2l0ZWRCeUNvdW50IjowfX0sInF1b3RlIjoiRHVlIHRvIHRoZSBkZWNyZWFzZSBpbiB0aGUgdmFsdWUgb2YgZXhwb3J0cyBvZiBnb29kcyBhbmQgc2VydmljZXMgdG8gZm9yZWlnbiBjb3VudHJpZXMsIHRoZXJlIGlzIGEgc3RyYXRlZ3kgdG8gc29sdmUgdGhlIHByb2JsZW0gYnkgY3JlYXRpbmcgaW50ZXJuYXRpb25hbGx5IGFjY2VwdGVkIHByb2R1Y3Qgc3RhbmRhcmRzLiBUaGVyZWZvcmUsIG5ldyB0ZWNobm9sb2dpZXMgaGF2ZSBiZWVuIGRldmVsb3BlZCBhbmQgdXNlZCBpbiB0aGUgZnJvemVuIHNlYWZvb2QgaW5kdXN0cnkgZm9yIGV4cG9ydCwgbmFtZWx5IFBsYXNtYSBUZWNobm9sb2d5LiBQbGFzbWEgdGVjaG5vbG9neSBpcyBnYWluaW5nIHNpZ25p77%2B9Y2FudCBhdHRlbnRpb24gYW1vbmcgcmVzZWFyY2hlcnMgYXMgYW4gaW5ub3ZhdGl2ZSBhbHRlcm5hdGl2ZSB0ZWNobm9sb2d5IGZvciB2YXJpb3VzIHJlc2VhcmNoIGFwcGxpY2F0aW9ucy4gT25lIG9mIHRoZSBrZXkgaGlnaGxpZ2h0cyBvZiBQbGFzbWEgVGVjaG5vbG9neSBpcyBpdHMgQ2xlYW4gVGVjaG5vbG9neSBhc3BlY3QsIHdoaWNoIGZvY3VzZXMgb24gZW5oYW5jaW5nIGFuZCByZe%2B%2FvW5pbmcgdGhlIHN1cmZhY2UgcHJvcGVydGllcyBvZiB0aGUgdGFyZ2V0IHByb2R1Y3Qgd2l0aG91dCBhbHRlcmluZyBpdHMgY2hlbWljYWwgY29tcG9zaXRpb24uIEJ5IGNvYXRpbmcgcGxhc21hIG9uIHRoZSBzdXJmYWNlLCB0aGUgdGVjaG5vbG9neSBjYW4gZWZmZWN0aXZlbHkgbWFuYWdlIGltcHVyaXRpZXMgcHJlc2VudCBvbiByYXcgbWF0ZXJpYWxzLCBvZmZlcmluZyBhbiBhZGRpdGlvbmFsIG1ldGhvZCB0byBpbXByb3ZlIHRoZSBl77%2B9Y2FjeSBvZiBpbmhpYml0aW5nIHRoZSBncm93dGggb2YgbWljcm9vcmdhbmlzbXMgaW4gYXF1YXRpYyBmb29kLiBUaGlzIGlzIHBhcnRpY3VsYXJseSBjcnVjaWFsIGluIG1lZXRpbmcgdGhlIHN0cmljdCByZXF1aXJlbWVudHMgZm9yIGV4cG9ydGluZyBnb29kcyBiZXR3ZWVuIGNvdW50cmllcy4gQ29sZCBwbGFzbWEgaXMgYSBuZXcgbWV0aG9kIHRoYXQgaGFzIGJlZW4gdGhvcm91Z2hseSByZXNlYXJjaGVkIGZvciBmb29kIHNhZmV0eSAoWHUgZXQgYWwuLCAyMDE2KS4gRXNwZWNpYWxseSBwbGFzbWEgYWN0aXZhdGVkIHdhdGVyIChQQVcpIGlzIGZyZXF1ZW50bHkgcHJvZHVjZWQgYnkgZGlzY2hhcmdpbmcgY29sZCBwbGFzbWEgYWJvdmUgb3IgYmVuZWF0aCB0aGUgd2F0ZXIgc3VyZmFjZSwgd2hpY2ggcmVzdWx0cyBpbiB0aGUgZm9ybWF0aW9uIG9mIHNlY29uZGFyeSByZWFjdGl2ZSBzcGVjaWVzIHN1Y2ggYXMgaHlkcm94eWwgcmFkaWNhbCAoT0gpLCBoeWRyb2dlbiBwZXJveGlkZSAoSDJPMiksIHN1cGVyb3hpZGUsIHNpbmdsZSBveHlnZW4sIG96b25lLCBuaXRyYXRlLCBhbmQgbml0cmljIG94aWRlLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjIwLjY3MzIyMzQ5NTQ4MzQsImxlZnQiOjQuNTc1MTYyODg3NTczMjQyLCJ3aWR0aCI6ODkuMjMxNjQzNjc2NzU3ODEsImhlaWdodCI6MS43NzcxNjMzODYzNDQ5MDk3fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MjIuOTQ3OTg2NjAyNzgzMjAzLCJsZWZ0Ijo0LjU3NTE2Mjg4NzU3MzI0Miwid2lkdGgiOjg5Ljc4MzU5OTg1MzUxNTYyLCJoZWlnaHQiOjEuNzc3MTYzMzg2MzQ0OTA5N319LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjI1LjIyMjc2MzA2MTUyMzQzOCwibGVmdCI6NC41NzUxNjI4ODc1NzMyNDIsIndpZHRoIjo4OS40MjEzNzkwODkzNTU0NywiaGVpZ2h0Ijo0LjA1MTkyNTE4MjM0MjUyOX19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjMxLjI4ODgwODgyMjYzMTgzNiwibGVmdCI6NC41NzUxNjI4ODc1NzMyNDIsIndpZHRoIjo4NC44MTU5MTAzMzkzNTU0NywiaGVpZ2h0IjoxLjc3NzE2MzM4NjM0NDkwOTd9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjozMy41NjM1ODcxODg3MjA3LCJsZWZ0Ijo0LjU3NTE2Mjg4NzU3MzI0Miwid2lkdGgiOjkwLjc0NjY1ODMyNTE5NTMxLCJoZWlnaHQiOjQuMDUxOTI1MTgyMzQyNTI5fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MzguMTEzMTI0ODQ3NDEyMTEsImxlZnQiOjQuNTc1MTYyODg3NTczMjQyLCJ3aWR0aCI6ODcuMTQ2NDM4NTk4NjMyODEsImhlaWdodCI6NC4wNTE5MjUxODIzNDI1Mjl9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo0Mi42NjI2NjYzMjA4MDA3OCwibGVmdCI6NC41NzUxNjI4ODc1NzMyNDIsIndpZHRoIjo5MC42NDIyMTE5MTQwNjI1LCJoZWlnaHQiOjQuMDUxOTI1MTgyMzQyNTI5fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6NDcuMjEyMjAzOTc5NDkyMTksImxlZnQiOjQuNTc1MTYyODg3NTczMjQyLCJ3aWR0aCI6OTAuMjEyOTEzNTEzMTgzNiwiaGVpZ2h0Ijo0LjA1MTkyNTE4MjM0MjUyOX19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjUxLjc2MTc0MTYzODE4MzU5NCwibGVmdCI6NC41NzUxNjI4ODc1NzMyNDIsIndpZHRoIjo4NC4wMzg3NDk2OTQ4MjQyMiwiaGVpZ2h0IjoyLjA2MzczMjE0NzIxNjc5N319LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjU0LjIyNjA3MDQwNDA1MjczNCwibGVmdCI6NC41NzUxNjI4ODc1NzMyNDIsIndpZHRoIjo4NS40MjkyMTQ0Nzc1MzkwNiwiaGVpZ2h0IjoxLjc3NzE2MzM4NjM0NDkwOTd9fV19LCJjaHVua0lkIjoiZWJhMThiYzYtMWU3ZS00ZGUwLWE4YzEtNTQ2OGUyNzY1YzZjIiwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiMyJ9fV0%3D. This non-thermal approach operates at ambient temperatures, ensuring that the treatment does not induce thermal stress on aquatic species while maintaining high bactericidal efficiency and environmental friendliness (Zhang et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikh1YW5sYW4iLCJmYW1pbHkiOiJaaGFuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJKaWUiLCJmYW1pbHkiOiJXZWkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSG9uZ2ppZSIsImZhbWlseSI6Ilh2In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkltcmFuIiwiZmFtaWx5IjoiS2hhbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJRaW54aXUiLCJmYW1pbHkiOiJTdW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiWGlob25nIiwiZmFtaWx5IjoiWmhhbyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJKaWFsb25nIiwiZmFtaWx5IjoiR2FvIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNodWNoZW5nIiwiZmFtaWx5IjoiTGl1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNodWFpIiwiZmFtaWx5IjoiV2VpIn19XSwiaWRzIjp7Imlzc24iOlsiMjI5Ni04NjFYIl0sImlzYm4iOltdLCJkb2kiOiIxMC4zMzg5L2ZudXQuMjAyNC4xMzY1MjgyIiwicG1pZCI6IjM4NTE1NTI0Iiwib3BlbmFsZXgiOiJXNDM5MjU1ODIzMCIsIm9wZW5BY2Nlc3NJZCI6IjEzOTI4ODg4Mzk5MDMyNzU1MDUwIn0sInRpdGxlIjoiQmFjdGVyaWNpZGFsIGVmZmljYWN5IG9mIHBsYXNtYS1hY3RpdmF0ZWQgd2F0ZXIgYWdhaW5zdCBWaWJyaW8gcGFyYWhhZW1vbHl0aWN1cyBvbiBMaXRvcGVuYWV1cyB2YW5uYW1laSIsImFic3RyYWN0IjoiSW4gdGhpcyBzdHVkeSwgdGhlIGFudGltaWNyb2JpYWwgbWVjaGFuaXNtIG9mIHBsYXNtYS1hY3RpdmF0ZWQgd2F0ZXIgKFBBVykgYWdhaW5zdCIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzg5L2ZudXQuMjAyNC4xMzY1MjgyIiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5mcm9udGllcnNpbi5vcmcvYXJ0aWNsZXMvMTAuMzM4OS9mbnV0LjIwMjQuMTM2NTI4Mi9wZGY%2FaXNQdWJsaXNoZWRWMj1GYWxzZSIsImlzc3VlZERhdGUiOiIyMDI0LTAzLTA3IiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiRnJvbnRpZXJzIGluIE51dHJpdGlvbiIsInZvbHVtZSI6MTEsImltcGFjdEZhY3RvciI6IjUuNDAifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRnJvbnRpZXJzIE1lZGlhIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo0MSwiY2l0ZWRCeUNvdW50IjozfX0sInF1b3RlIjoiSG93ZXZlciwgdGhlc2UgbWV0aG9kcyBhcmUgdW5hYmxlIHRvIG92ZXJjb21lIHRoZSAgaXJyZWd1bGFyaXRpZXMgaW4gdGhlIGZvb2QgbWF0ZXJpYWwsIGFuZCB0aGUgZWZmZWN0aXZlIHN0ZXJpbGl6aW5nIHN1YnN0YW5jZXMgIGFyZSB1bmFibGUgdG8gcGVuZXRyYXRlIHRoZSB0aW55IGhvbGxvd3MgYW5kIHNsb3RzIGluIHRoZSBmb29kIG1hdGVyaWFsLCAgdGh1cyByZWR1Y2luZyB0aGUgZWZmZWN0aXZlbmVzcyBvZiBzdGVyaWxpemF0aW9uICgxMCkuIFRoZXJlZm9yZSwgYW4gIGFsdGVybmF0aXZlIG1ldGhvZCBpcyByZXF1aXJlZCB0byBvdmVyY29tZSB0aGVzZSBwcm9ibGVtcy4gUGxhc21hLWFjdGl2YXRlZCB3YXRlciAoUEFXKSBpcyBjb25zaWRlcmVkIGFzIGEgcHJvbWlzaW5nIGFsdGVybmF0aXZlIG5vbi10aGVybWFsIGFwcGxpY2F0aW9uIG9mIHBsYXNtYSB0ZWNobm9sb2d5LCB3aGljaCBpcyBwcm9kdWNlZCBieSB0cmVhdGluZyB3YXRlciB1c2luZyBwbGFzbWEtZ2VuZXJhdGlvbiBkZXZpY2VzICgxMSkuIEl0IGlzICBjYXBhYmxlIG9mIHN0ZXJpbGl6aW5nIHRocm91Z2ggb3hpZGF0aXZlIHN0cmVzcyBhbmQgcGh5c2ljYWwgZWZmZWN0cyBieSAgaW50ZXJhY3Rpbmcgd2l0aCBhY3RpdmUgc3Vic3RhbmNlcyBpbiBQQVcgKDEyKS4gVW5saWtlIG90aGVyICB0cmVhdG1lbnRzLCBQQVcgY291bGQgYmUgb3BlcmF0ZWQgaW4gYmF0Y2gtdHlwZSBmb3IgY29tbWVyY2lhbCAgcHJvY2Vzcy4gSW4gcmVjZW50IHllYXJzLCBpdCBoYXMgYmVlbiB3aWRlbHkgdXNlZCBmb3IgbWljcm9iaWFsICBpbmFjdGl2YXRpb24gZHVlIHRvIGl0cyBoaWdoIGJhY3RlcmljaWRhbCBlZmZpY2llbmN5LCBlbnZpcm9ubWVudGFsICBmcmllbmRsaW5lc3MsIGVhc2Ugb2YgcHJlcGFyYXRpb24sIGFuZCBjb3N0IGVmZmVjdGl2ZW5lc3MgKDEzKS4gIEZ1cnRoZXJtb3JlLCBQQVcgZXhoaWJpdHMgZmF2b3JhYmxlIHBlcm1lYWJpbGl0eSBjaGFyYWN0ZXJpc3RpY3MsICBlbmFibGluZyBpdCB0byBlZmZlY3RpdmVseSBpbmZpbHRyYXRlIG1pbnVzY3VsZSB2b2lkcyBhbmQgY3JldmljZXMgd2l0aGluICBmb29kIHN1YnN0YW5jZXMsIGNvbnNlcXVlbnRseSBlbmhhbmNpbmcgaXRzIGRpc2luZmVjdGFudCBwcm9wZXJ0aWVzICAoMTQpLiBJbiB0aGUgc3R1ZHkgYnkgR2FuIGV0IGFsLiAoMTUpLCBQQVcgd2FzIGVtcGxveWVkIHRvIHRyZWF0ICBFc2NoZXJpY2hpYSBjb2xpIGFuZCBTdGFwaHlsb2NvY2N1cyBhdXJldXMgb24gYmx1ZWJlcnJpZXMsICBkZW1vbnN0cmF0aW5nIGl0cyBlZmZlY3RpdmVuZXNzIGluIHJlZHVjaW5nIG1pY3Jvb3JnYW5pc21zLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI1LjA3NjIxMzgzNjY2OTkyMiwibGVmdCI6OC4zMzE2NDc4NzI5MjQ4MDUsIndpZHRoIjo0MC44MjA4OTIzMzM5ODQzNzUsImhlaWdodCI6MS40NDIxMTMxNjEwODcwMzYxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MjYuNTAxNTc3Mzc3MzE5MzM2LCJsZWZ0Ijo4LjMzMTY0Nzg3MjkyNDgwNSwid2lkdGgiOjQwLjgxNDkxODUxODA2NjQwNiwiaGVpZ2h0IjoxLjQ0MjExMzE2MTA4NzAzNjF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyNy45MjY5NDI4MjUzMTczODMsImxlZnQiOjguMzMxNjQ3ODcyOTI0ODA1LCJ3aWR0aCI6NDAuODI5ODUzMDU3ODYxMzMsImhlaWdodCI6MS40NDIxMTMxNjEwODcwMzYxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MjkuMzUyMzA2MzY1OTY2Nzk3LCJsZWZ0Ijo4LjMzMTY0Nzg3MjkyNDgwNSwid2lkdGgiOjQwLjgyMjU1NTU0MTk5MjE5LCJoZWlnaHQiOjEuNDQyMTExMjUzNzM4NDAzM319LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMwLjc3NzY3MTgxMzk2NDg0NCwibGVmdCI6OC4zMzE2NDc4NzI5MjQ4MDUsIndpZHRoIjozMy4zMTM0ODQxOTE4OTQ1MywiaGVpZ2h0IjoxLjQ0MjExMTI1MzczODQwMzN9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozMi4yMDMwMzM0NDcyNjU2MjUsImxlZnQiOjEwLjcxMjU2MDY1MzY4NjUyMywid2lkdGgiOjM4LjQ0NTcxMzA0MzIxMjg5LCJoZWlnaHQiOjEuNDQyMTExMjUzNzM4NDAzM319LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMzLjYyODM5ODg5NTI2MzY3LCJsZWZ0Ijo4LjMzMTY0Nzg3MjkyNDgwNSwid2lkdGgiOjQwLjgyMzEyMDExNzE4NzUsImhlaWdodCI6MS40NDIxMTEyNTM3Mzg0MDMzfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzUuMDUzNzY0MzQzMjYxNzIsImxlZnQiOjguMzMxNjQ3ODcyOTI0ODA1LCJ3aWR0aCI6NDAuODEzMzY1OTM2Mjc5MywiaGVpZ2h0IjoxLjQ0MjExMTI1MzczODQwMzN9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNi40NzkxMjU5NzY1NjI1LCJsZWZ0Ijo4LjMzMTY0Nzg3MjkyNDgwNSwid2lkdGgiOjQwLjgyMTY4OTYwNTcxMjg5LCJoZWlnaHQiOjEuNDQyMTExMjUzNzM4NDAzM319LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjM3LjkwNDQ5MTQyNDU2MDU1LCJsZWZ0Ijo4LjMzMTY0Nzg3MjkyNDgwNSwid2lkdGgiOjQwLjgyMjA0ODE4NzI1NTg2LCJoZWlnaHQiOjEuNDQyMTExMjUzNzM4NDAzM319LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjM5LjMyOTg1MzA1Nzg2MTMzLCJsZWZ0Ijo4LjMzMTY0Nzg3MjkyNDgwNSwid2lkdGgiOjQwLjgxOTAwNDA1ODgzNzg5LCJoZWlnaHQiOjEuNDQyMTExMjUzNzM4NDAzM319LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQwLjc1NTIxODUwNTg1OTM3NSwibGVmdCI6OC4zMzE2NDc4NzI5MjQ4MDUsIndpZHRoIjo0MC44MjczOTYzOTI4MjIyNjYsImhlaWdodCI6MS40NDIxMTEyNTM3Mzg0MDMzfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDIuMTgwNTgzOTUzODU3NDIsImxlZnQiOjguMzMxNjQ3ODcyOTI0ODA1LCJ3aWR0aCI6NDAuODI3NTc1NjgzNTkzNzUsImhlaWdodCI6MS40NDIxMTEyNTM3Mzg0MDMzfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDMuNjA1OTQ5NDAxODU1NDcsImxlZnQiOjguMzMxNjQ3ODcyOTI0ODA1LCJ3aWR0aCI6NDAuODIyNzQ2Mjc2ODU1NDcsImhlaWdodCI6MS40NDIxMTEyNTM3Mzg0MDMzfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDUuMDMxMzExMDM1MTU2MjUsImxlZnQiOjguMzMxNjQ3ODcyOTI0ODA1LCJ3aWR0aCI6NDAuODI2MTkwOTQ4NDg2MzMsImhlaWdodCI6MS40NDIxMTEyNTM3Mzg0MDMzfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDYuNDU2NjcyNjY4NDU3MDMsImxlZnQiOjguMzMxNjQ3ODcyOTI0ODA1LCJ3aWR0aCI6NDAuODEyMjM2Nzg1ODg4NjcsImhlaWdodCI6MS40NDIxMTEyNTM3Mzg0MDMzfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDcuODgyMDM4MTE2NDU1MDgsImxlZnQiOjguMzMxNjQ3ODcyOTI0ODA1LCJ3aWR0aCI6NDAuODE4NDQzMjk4MzM5ODQ0LCJoZWlnaHQiOjEuNDQyMTExMjUzNzM4NDAzM319LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQ5LjMwNzQwMzU2NDQ1MzEyNSwibGVmdCI6OC4zMzE2NDc4NzI5MjQ4MDUsIndpZHRoIjo0MC44MjI3MjcyMDMzNjkxNCwiaGVpZ2h0IjoxLjQ0MjExMTI1MzczODQwMzN9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1MC43MTg4NjgyNTU2MTUyMzQsImxlZnQiOjguMzMxNjQ3ODcyOTI0ODA1LCJ3aWR0aCI6NDAuODIxNjU1MjczNDM3NSwiaGVpZ2h0IjoxLjQ1NjAwOTE0OTU1MTM5MTZ9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1Mi4xNTgxMzA2NDU3NTE5NSwibGVmdCI6OC4zMzE2NDc4NzI5MjQ4MDUsIndpZHRoIjo0MC44MjQ2Njg4ODQyNzczNDQsImhlaWdodCI6MS40NDIxMTEyNTM3Mzg0MDMzfX1dfSwiY2h1bmtJZCI6IjM1YzE0ODg4LTY0NjYtNDAxYy05ZGI4LTU4MjcxYmZiMjAwZCIsImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjIifX1d.

Thus, the objective of this review considered to provide a general overview of nanobubbles and plasma water technologies and its application in aquaculture. The submission will also discuss the implications of nanobubble & plasma water technology operation and its potential benefits, challenges, and future research directions to develop sustainable aquaculture. By integrating these advanced oxidation technologies, producers can minimize the environmental footprint of aquaculture operations while enhancing productivity and disease resistance (Hadinoto et al., 2022; Nghĩa et al., 2022)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5ndXnhu4VuIEjhu691IiwiZmFtaWx5IjoiTmdoxKlhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5ndXllbiBUaGkiLCJmYW1pbHkiOiJOZ3V5ZW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUGhhbiBUcuG7jW5nIiwiZmFtaWx5IjoiQsOsbmgifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTGUgVGhpIiwiZmFtaWx5IjoiTWF5In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRvbmcgVHJhbiIsImZhbWlseSI6Ikh1eSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQaOG6oW0gVGjDoWkiLCJmYW1pbHkiOiJHaWFuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTb3BoaWUiLCJmYW1pbHkiOiJTdOKAkEhpbGFpcmUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUGhhbiBUaGkiLCJmYW1pbHkiOiJWYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMjM0LTE3NDkiLCIyMjM0LTE3NTciXSwiaXNibiI6W10sImRvaSI6IjEwLjQ3ODUzL2Zhcy4yMDIyLmUzOSIsIm9wZW5hbGV4IjoiVzQyOTQ1OTc1NTQiLCJvcGVuQWNjZXNzSWQiOiIxMzQ4NTgzNDA1NjEzOTM2NDAzIn0sInRpdGxlIjoiRWZmZWN0IG9mIG5hbm9idWJibGVzIChveHlnZW4sIG96b25lKSBvbiB0aGUgUGFjaWZpYyB3aGl0ZSBzaHJpbXAgKFBlbmFldXMgdmFubmFtZWkpLCBWaWJyaW8gcGFyYWhhZW1vbHl0aWN1cyBhbmQgd2F0ZXIgcXVhbGl0eSB1bmRlciBsYWIgY29uZGl0aW9ucyIsImFic3RyYWN0IjoiVGhpcyBzdHVkeSBhc3Nlc3NlZCB0aGUgZWZmZWN0cyBvZiBveHlnZW4gYW5kIG96b25lIG5hbm9idWJibGVzIG9uIGdpbGwgbW9ycGhvbG9neSwgd2VpZ2h0IGdhaW4sIGFuZCBtb3J0YWxpdHkgb2YgUGFjaWZpYyB3aGl0ZSBzaHJpbXAgKFBlbmFldXMgdmFubmFtZWkpLCBhcyB3ZWxsIGFzIHRoZSBsZXZlbCBvZiBWaWJyaW8gcGFyYWhhZW1vbHl0aWN1cyBhbmQgd2F0ZXIgcXVhbGl0eSBvZiBzaHJpbXAgY3VsdHVyZSB0YW5rcyB1bmRlciBsYWIgY29uZGl0aW9ucy4gVHdvIGV4cGVyaW1lbnRzIHdlcmUgY2FycmllZCBvdXQgd2l0aCBveHlnZW4gbWFjcm9idWJibGUsIG96b25lIG1hY3JvYnViYmxlLCBveHlnZW4gbmFub2J1YmJsZSwgb3pvbmUgbmFub2J1YmJsZSwgYW5kIGNvbnRyb2wgdHJlYXRtZW50cyAoYWlyLXN0b25lIG1hY3JvYnViYmxlKS4gRXhwZXJpbWVudHMgd2VyZSBkb25lIGluIHRyaXBsaWNhdGUgaW4gMTAwIEwgdGFua3Mgd2l0aCAxNeKAsCBzYWxpbmUgd2F0ZXIsIGFuZCAyMCBzaHJpbXAgcGVyIHRhbmsuIFRhbmtzIGluIEV4cGVyaW1lbnQgMSB3ZXJlIG5vdCBpbm9jdWxhdGVkIHdpdGggYmFjdGVyaWE7IHRhbmtzIGluIEV4cGVyaW1lbnQgMiB3ZXJlIGlub2N1bGF0ZWQgd2l0aCBWLiBwYXJhaGFlbW9seXRpY3VzIGF0IGEgY29uY2VudHJhdGlvbiBvZiAxMDYgQ0ZVL21MLiBUaGUgcmVzdWx0cyByZXZlYWxlZCB0aGF0IHNob3J0IHRyZWF0bWVudHMgd2l0aCBvem9uZSBuYW5vYnViYmxlcyBoYWQgbWluaW1hbCBpbXBhY3Qgb24gc2hyaW1wIGdpbGxzLCBtb3J0YWxpdHksIGFuZCBncm93dGggcmF0ZXMsIHJlZHVjZWQgVi4gcGFyYWhhZW1vbHl0aWN1cyBjb25jZW50cmF0aW9uIGluIHdhdGVyIGNvbXBhcmVkIHRvIHRoZSBvdGhlciBncm91cHMsIGFuZCBpbXByb3ZlIHdhdGVyIHF1YWxpdHkuIFRoZXNlIGxhYm9yYXRvcnkgcmVzdWx0cyBpbmRpY2F0ZSB0aGF0IG96b25lIG5hbm9idWJibGUgdHJlYXRtZW50IG1heSBiZSB1c2VmdWwgZm9yIGNvbnRyb2xsaW5nIFYuIHBhcmFoYWVtb2x5dGljdXMuIE1vcmUgd29yayBpcyBuZWVkZWQgdG8gZmluZCB0aGUgYmVzdCBwcm90b2NvbCB0byBhcHBseSB0aGUgdGVjaG5vbG9neSBvbiBhIGNvbW1lcmNpYWwgc2NhbGUuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjQ3ODUzL2Zhcy4yMDIyLmUzOSIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cuZS1mYXMub3JnL2Rvd25sb2FkL2Rvd25sb2FkX3BkZj9waWQ9ZmFzLTI1LTgtNDI5IiwiaXNzdWVkRGF0ZSI6IjIwMjItMDgtMDEiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA5IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJGaXNoZXJpZXMgYW5kIGFxdWF0aWMgc2NpZW5jZXMiLCJpc3N1ZSI6OCwidm9sdW1lIjoyNSwiaW1wYWN0RmFjdG9yIjoiMS4zNCJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjQyOSwibGFzdFBhZ2UiOjQ0MH19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJTcHJpbmdlciBTY2llbmNlK0J1c2luZXNzIE1lZGlhIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjozMCwiY2l0ZWRCeUNvdW50IjoxM319LCJxdW90ZSI6IkFic3RyYWN0IFRoaXMgc3R1ZHkgYXNzZXNzZWQgdGhlIGVmZmVjdHMgb2Ygb3h5Z2VuIGFuZCBvem9uZSBuYW5vYnViYmxlcyBvbiBnaWxsIG1vcnBob2xvZ3ksIHdlaWdodCBnYWluLCBhbmQgbW9ydGFsaXR5IG9mIFBhY2lmaWMgd2hpdGUgc2hyaW1wIChQZW5hZXVzIHZhbm5hbWVpKSwgYXMgd2VsbCBhcyB0aGUgbGV2ZWwgb2YgVmlicmlvIHBhcmFoYWVtb2x5dGljdXMgYW5kIHdhdGVyIHF1YWxpdHkgb2Ygc2hyaW1wIGN1bHR1cmUgdGFua3MgdW4tZGVyIGxhYiBjb25kaXRpb25zLiBUd28gZXhwZXJpbWVudHMgd2VyZSBjYXJyaWVkIG91dCB3aXRoIG94eWdlbiBtYWNyb2J1YmJsZSwgb3pvbmUgbWFjcm9idWJibGUsIG94eWdlbiBuYW5vYnViYmxlLCAgb3pvbmUgbmFub2J1YmJsZSwgYW5kIGNvbnRyb2wgdHJlYXRtZW50cyAoYWlyLXN0b25lIG1hY3JvYnViYmxlKS4gRXhwZXJpbWVudHMgd2VyZSBkb25lIGluIHRyaXBsaWNhdGUgaW4gMTAwIEwgdGFua3Mgd2l0aCAgMTXigLAgc2FsaW5lIHdhdGVyLCBhbmQgMjAgc2hyaW1wIHBlciB0YW5rLiBUYW5rcyBpbiBFeHBlcmltZW50IDEgd2VyZSBub3QgaW5vY3VsYXRlZCB3aXRoIGJhY3RlcmlhOyB0YW5rcyBpbiBFeHBlcmltZW50IDIgd2VyZSAgaW5vY3VsYXRlZCB3aXRoIFYuIHBhcmFoYWVtb2x5dGljdXMgYXQgYSBjb25jZW50cmF0aW9uIG9mIDEwIDYgQ0ZVL21MLiBUaGUgcmVzdWx0cyByZXZlYWxlZCB0aGF0IHNob3J0IHRyZWF0bWVudHMgd2l0aCBvem9uZSBuYS1ub2J1YmJsZXMgaGFkIG1pbmltYWwgaW1wYWN0IG9uIHNocmltcCBnaWxscywgbW9ydGFsaXR5LCBhbmQgZ3Jvd3RoIHJhdGVzLCByZWR1Y2VkIFYuIHBhcmFoYWVtb2x5dGljdXMgY29uY2VudHJhdGlvbiBpbiB3YXRlciAgY29tcGFyZWQgdG8gdGhlIG90aGVyIGdyb3VwcywgYW5kIGltcHJvdmUgd2F0ZXIgcXVhbGl0eS4gVGhlc2UgbGFib3JhdG9yeSByZXN1bHRzIGluZGljYXRlIHRoYXQgb3pvbmUgbmFub2J1YmJsZSB0cmVhdG1lbnQgIG1heSBiZSB1c2VmdWwgZm9yIGNvbnRyb2xsaW5nIFYuIHBhcmFoYWVtb2x5dGljdXMuIE1vcmUgd29yayBpcyBuZWVkZWQgdG8gZmluZCB0aGUgYmVzdCBwcm90b2NvbCB0byBhcHBseSB0aGUgdGVjaG5vbG9neSBvbiBhICBjb21tZXJjaWFsIHNjYWxlLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjQ0LjI0NDAyNjE4NDA4MjAzLCJsZWZ0Ijo5LjYyNDU5NDY4ODQxNTUyNywid2lkdGgiOjgxLjIwNzEzODA2MTUyMzQ0LCJoZWlnaHQiOjE5LjE5OTkyNjM3NjM0Mjc3M319XX0sImNodW5rSWQiOiIwMzg1M2ViMS0zMDlkLTQ3MTEtYTU1Ny04YzZlYTMxOTUxMTciLCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI0MjkifX0seyJvcmlnaW4iOjUsIndvcmsiOnsidHlwZSI6OCwic3VidHlwZSI6eyJvbmVvZktpbmQiOiJhcnRpY2xlVHlwZSIsImFydGljbGVUeXBlIjoxfSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiS29lbnRhZGkiLCJmYW1pbHkiOiJIYWRpbm90byJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJOLlIuSC4iLCJmYW1pbHkiOiJSYW8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSmF2aWVyYSBCYXJyYWxlcyIsImZhbWlseSI6IkFzdG9yZ2EifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUmVud3UiLCJmYW1pbHkiOiJaaG91In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkpvYW5uYSIsImZhbWlseSI6IkJpYXppayJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJUaWFucWkiLCJmYW1pbHkiOiJaaGFuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQYXRyaWNrIEouIiwiZmFtaWx5IjoiQ3VsbGVuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlN0dWFydCBXLiIsImZhbWlseSI6IlByZXNjb3R0In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlJpdGEgSy4iLCJmYW1pbHkiOiJIZW5kZXJzb24ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiRnJhbmNpc2NvIEouIiwiZmFtaWx5IjoiVHJ1amlsbG8ifX1dLCJpZHMiOnsiaXNzbiI6WyIxNTU2LTUwNjgiXSwiaXNibiI6W10sImRvaSI6IjEwLjIxMzkvc3Nybi40MTE1MTY3Iiwib3BlbmFsZXgiOiJXNDI4NTI4Njc1MiIsImV4YSI6Imh0dHBzOi8vcGFwZXJzLnNzcm4uY29tL3NvbDMvcGFwZXJzLmNmbT9hYnN0cmFjdF9pZD00MTE1MTY3In0sInRpdGxlIjoiSHlicmlkIFBsYXNtYS1CdWJibGUgRGlzY2hhcmdlcyBmb3IgRW5lcmd5LUVmZmljaWVudCBQcm9kdWN0aW9uIG9mIFBsYXNtYS1BY3RpdmF0ZWQgV2F0ZXIiLCJhYnN0cmFjdCI6IlBsYXNtYS1hY3RpdmF0ZWQgd2F0ZXIgKFBBVykgaXMgYW4gZW1lcmdpbmcsIGNvc3QtZWZmaWNpZW50IGFuZCBlbnZpcm9ubWVudGFsbHkgZnJpZW5kbHkgZGlzaW5mZWN0YW50LCB3aXRoIGEgcG90ZW50aWFsIHRvIHN1YnN0aXR1dGUgY29udmVudGlvbmFsIGNoZW1pY2FsIGFnZW50cyB1c2VkIGZvciBhZ3JpY3VsdHVyZSwgbWVkaWNhbCBhbmQgZm9vZCBhcHBsaWNhdGlvbnMuIFBBVyBpcyBwcm9kdWNlZCBmcm9tIHRoZSBpbnRlcmFjdGlvbnMgYmV0d2VlbiBub24tdGhlcm1hbCBwbGFzbWFzIHdpdGggd2F0ZXIsIGFuZCBpdCBjb250YWlucyBoaWdobHkgYmlvY2hlbWljYWxseSBhY3RpdmUgcmVhY3RpdmUgb3h5Z2VuIGFuZCBuaXRyb2dlbiBzcGVjaWVzIChST05TKS4gVGhlIGNoZW1pY2FsIGNvbXBvc2l0aW9uIG9mIHJlYWN0aXZlIHNwZWNpZXMgaW4gUEFXIGlzIG5vdCBvbmx5IGFmZmVjdGVkIGJ5IHRoZSBpbnB1dCB2b2x0YWdlIGFuZCB0aGUgbGlxdWlkIGNvbXBvc2l0aW9uIGJ1dCBhbHNvIGJ5IHRoZSByZWFjdG9yIGRlc2lnbiBjaGFyYWN0ZXJpc3RpY3MgaW5jbHVkaW5nIHRoZSBlbGVjdHJvZGUgY29uZmlndXJhdGlvbiBhbmQgdGhlIHBsYXNtYSB0byBsaXF1aWQgY29udGFjdCBhcnJhbmdlbWVudCwgc3VjaCBhcyBwbGFzbWEtYnViYmxlIGRpc2NoYXJnZXMuIEluIHRoaXMgd29yaywgUEFXIHdhcyBnZW5lcmF0ZWQgd2l0aCBhIG5ld2x5IGRlc2lnbmVkIGh5YnJpZCBhaXIgcGxhc21hLWJ1YmJsZSBkaXNjaGFyZ2UgKEhQRCkgcmVhY3RvciB0aGF0IGNvbWJpbmVzIGEgcGluLXRvLWxpcXVpZCBkaXNjaGFyZ2UgYW5kIGEgcGxhc21hLWJ1YmJsZSBkaXNjaGFyZ2UuIFRoZSBQQVcgZ2VuZXJhdGVkIHZpYSB0aGUgSERQIHJlYWN0b3IgYWNoaWV2ZWQgYW4gZXhjZXB0aW9uYWwgUk9OUyBlbmVyZ3kgZWZmaWNpZW5jeSBvZiA4LjgzIGfCt2tXIC0xwrdoIC0xIGFuZCBhIGhpZ2ggaW5hY3RpdmF0aW9uIGFnYWluc3QgRXNjaGVyaWNoaWEgY29saSBvZiA1LjE4LWxvZzEwIHJlZHVjdGlvbiBpbiAzMHMgb2YgY29udGFjdCB0aW1lLCBvdXRwZXJmb3JtaW5nIHJlcG9ydGVkIGxpdGVyYXR1cmUgc3R1ZGllcywgd2hpY2ggcmVxdWlyZSBtaW51dGVzIG9yIGhvdXJzIHRvIGFjaGlldmUgc2ltaWxhciBpbmFjdGl2YXRpb24uIFRoZSBlZmZlY3Qgb2YgdGhlIG51bWJlciBhbmQgc2l6ZSBvZiBvcmlmaWNlcyBpbiB0aGUgYnViYmxlIGNvbHVtbiBvZiB0aGUgSFBEIHJlYWN0b3Igd2FzIHN0dWRpZWQsIGFzIHdlbGwgYXMgdGhlIGVmZmVjdCBvZiB0aGUgbGlxdWlkIHZvbHVtZSAoZnJvbSAwLjUgdG8gMiBMKSwgYW5kIHNhbGluaXR5IChmcm9tIDAgdG8gOG1NIE5hQ2wpLiBUaGUgRXNjaGVyaWNoaWEgY29saSBpbiBQQVcgd2FzIGZ1cnRoZXIgYW5hbHlzZWQgYnkgVEVNIGFuZCBETkEgbGVha2FnZSBzaG93aW5nIHRoYXQgUEFXIGRhbWFnZWQgdGhlIGNlbGwgbWVtYnJhbmUuIFRoaXMgaHlicmlkIGFpciBwbGFzbWEtYnViYmxlIGRpc2NoYXJnZSByZWFjdG9yIGNhbiBiZSB0aGUgYmFzZSBvZiBzY2FsZSB1cCBzeXN0ZW1zIHRvIHByb2R1Y2UgZGlzaW5mZWN0YW50IFBBVyBmb3IgdGhlIGFncmljdWx0dXJlLCBiaW9tZWRpY2FsLCBmb29kIGFuZCB3YXRlciB0cmVhdG1lbnQgaW5kdXN0cmllcy4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjEzOS9zc3JuLjQxMTUxNjciLCJpc3N1ZWREYXRlIjoiMjAyMi0wMS0wMSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IlNTUk4gRWxlY3Ryb25pYyBKb3VybmFsIiwiaW1wYWN0RmFjdG9yIjoiMC4zNyJ9LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJSRUxYIEdyb3VwIChOZXRoZXJsYW5kcykifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjY3LCJjaXRlZEJ5Q291bnQiOjJ9fSwicXVvdGUiOiJQbGFzbWEtYWN0aXZhdGVkIHdhdGVyIChQQVcpIGlzIGFuIGVtZXJnaW5nLCBjb3N0LWVmZmljaWVudCBhbmQgZW52aXJvbm1lbnRhbGx5IGZyaWVuZGx5IGRpc2luZmVjdGFudCwgd2l0aCBhIHBvdGVudGlhbCB0byBzdWJzdGl0dXRlIGNvbnZlbnRpb25hbCBjaGVtaWNhbCBhZ2VudHMgdXNlZCBmb3IgYWdyaWN1bHR1cmUsIG1lZGljYWwgYW5kIGZvb2QgYXBwbGljYXRpb25zLiBQQVcgaXMgcHJvZHVjZWQgZnJvbSB0aGUgaW50ZXJhY3Rpb25zIGJldHdlZW4gbm9uLXRoZXJtYWwgcGxhc21hcyB3aXRoIHdhdGVyLCBhbmQgaXQgY29udGFpbnMgaGlnaGx5IGJpb2NoZW1pY2FsbHkgYWN0aXZlIHJlYWN0aXZlIG94eWdlbiBhbmQgbml0cm9nZW4gc3BlY2llcyAoUk9OUykuIFRoZSBjaGVtaWNhbCBjb21wb3NpdGlvbiBvZiByZWFjdGl2ZSBzcGVjaWVzIGluIFBBVyBpcyBub3Qgb25seSBhZmZlY3RlZCBieSB0aGUgaW5wdXQgdm9sdGFnZSBhbmQgdGhlIGxpcXVpZCBjb21wb3NpdGlvbiBidXQgYWxzbyBieSB0aGUgcmVhY3RvciBkZXNpZ24gY2hhcmFjdGVyaXN0aWNzIGluY2x1ZGluZyB0aGUgZWxlY3Ryb2RlIGNvbmZpZ3VyYXRpb24gYW5kIHRoZSBwbGFzbWEgdG8gbGlxdWlkIGNvbnRhY3QgYXJyYW5nZW1lbnQsIHN1Y2ggYXMgcGxhc21hLWJ1YmJsZSBkaXNjaGFyZ2VzLiIsImV4dGVybmFsQ2FjaGVJZCI6ImFjZWQ4NTFmLTkwYjAtMzUwMy1hZGVkLWIzYTg4NmQ2MmI5OSJ9XQ%3D%3D. The integration of these advanced oxidation processes represents a paradigm shift toward precision aquaculture, where water quality is dynamically controlled through physical and chemical means rather than traditional chemical inputs (Thirumdas et al., 2018; Zorzi et al., 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlJvaGl0IiwiZmFtaWx5IjoiVGhpcnVtZGFzIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFuamluZXl1bHUiLCJmYW1pbHkiOiJLb3RoYWtvdGEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiVWRheSBTLiIsImZhbWlseSI6IkFubmFwdXJlIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkthbGlyYW1lc2giLCJmYW1pbHkiOiJTaWxpdmVydSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJSZW5hbGQiLCJmYW1pbHkiOiJCbHVuZGVsbCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJSdWJlbiIsImZhbWlseSI6IkdhdHQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiVmFzaWxpcyIsImZhbWlseSI6IlZhbGRyYW1pZGlzIn19XSwiaWRzIjp7Imlzc24iOlsiMDkyNC0yMjQ0IiwiMTg3OS0zMDUzIl0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDE2L2oudGlmcy4yMDE4LjA1LjAwNyIsIm1hZyI6IjI3OTk2MzM5MTciLCJvcGVuYWxleCI6IlcyNzk5NjMzOTE3IiwiZXhhIjoiaHR0cHM6Ly93d3cuc2NpZW5jZWRpcmVjdC5jb20vc2NpZW5jZS9hcnRpY2xlL2Ficy9waWkvUzA5MjQyMjQ0MTczMDU4NzMifSwidGl0bGUiOiJQbGFzbWEgYWN0aXZhdGVkIHdhdGVyIChQQVcpOiBDaGVtaXN0cnksIHBoeXNpY28tY2hlbWljYWwgcHJvcGVydGllcywgYXBwbGljYXRpb25zIGluIGZvb2QgYW5kIGFncmljdWx0dXJlIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjEwMTYvai50aWZzLjIwMTguMDUuMDA3IiwiaXNzdWVkRGF0ZSI6IjIwMTgtMDUtMDQiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTEwIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJUcmVuZHMgaW4gRm9vZCBTY2llbmNlICYgVGVjaG5vbG9neSIsInZvbHVtZSI6NzcsImltcGFjdEZhY3RvciI6IjE2LjE4In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MjEsImxhc3RQYWdlIjozMX19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJFbHNldmllciBCViJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NzksImNpdGVkQnlDb3VudCI6NzY5fX0sInF1b3RlIjoiQXV0aG9yIGxpbmtzIG9wZW4gb3ZlcmxheSBwYW5lbFJvaGl0VGhpcnVtZGFzYSwgQW5qaW5lbHl1bHVLb3RoYWtvdGFiLCBbVWRheUFubmFwdXJlY10oaHR0cHM6Ly93d3cuc2NpZW5jZWRpcmVjdC5jb20vYXV0aG9yLzY1MDc2NjY3NTYvdWRheS1zaHJpcmFtcmFvLWFubmFwdXJlKSwgS2FsaXJhbWVzaFNpbGl2ZXJ1ZCwgUmVuYWxkQmx1bmRlbGxlZiwgUnViZW5HYXR0ZiwgVmFzaWxpcyBQLlZhbGRyYW1pZGlzZ2ggQ29sZCBwbGFzbWEgaXMgYW4gZW1lcmdpbmcgbm9uLXRoZXJtYWwgZGlzaW5mZWN0aW9uIGFuZCBzdXJmYWNlIG1vZGlmaWNhdGlvbiB0ZWNobm9sb2d5IHdoaWNoIGlzIGNoZW1pY2FsIGZyZWUsIGFuZCBlY28tZnJpZW5kbHkuIFBsYXNtYSB0cmVhdG1lbnQgb2Ygd2F0ZXIsIHRlcm1lZCBhcyBwbGFzbWEgYWN0aXZhdGVkIHdhdGVyIChQQVcpLCBjcmVhdGVzIGFuIGFjaWRpYyBlbnZpcm9ubWVudCB3aGljaCByZXN1bHRzIGluIGNoYW5nZXMgb2YgdGhlIHJlZG94IHBvdGVudGlhbCwgY29uZHVjdGl2aXR5IGFuZCBpbiB0aGUgZm9ybWF0aW9uIG9mIHJlYWN0aXZlIG94eWdlbiAoUk9TKSBhbmQgbml0cm9nZW4gc3BlY2llcyAoUk5TKS4gQXMgYSByZXN1bHQsIFBBVyBoYXMgZGlmZmVyZW50IGNoZW1pY2FsIGNvbXBvc2l0aW9uIHRoYW4gd2F0ZXIgYW5kIGNhbiBzZXJ2ZSBhcyBhbiBhbHRlcm5hdGl2ZSBtZXRob2QgZm9yIG1pY3JvYmlhbCBkaXNpbmZlY3Rpb24uIiwiZXh0ZXJuYWxDYWNoZUlkIjoiMGEzZDc5NzAtYmI4My0zOTM0LWFlMWEtMjQwY2M3ZTdmZjdhIn0seyJvcmlnaW4iOjQsIndvcmsiOnsidHlwZSI6OCwic3VidHlwZSI6eyJvbmVvZktpbmQiOiJhcnRpY2xlVHlwZSIsImFydGljbGVUeXBlIjoxfSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiVml0dG9yaW8iLCJmYW1pbHkiOiJab3J6aSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBbmRyZWEiLCJmYW1pbHkiOiJCZXJ0aW5pIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFuZHJldyIsImZhbWlseSI6IlJvYmVydHNvbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBbm5hY2hpYXJhIiwiZmFtaWx5IjoiQmVyYXJkaW5lbGxpIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikxlb25hcmRvIiwiZmFtaWx5IjoiUGFsbWlzYW5vIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkZyYW5jZXNjbyIsImZhbWlseSI6IlBhcnJpbm8ifX1dLCJpZHMiOnsiaXNzbiI6WyIyNDY4LTgyMzEiLCIyNDY4LTgyNzQiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMTYvai5tY2F0LjIwMjMuMTEzNjE2Iiwib3BlbmFsZXgiOiJXNDM4Nzk2Mjg0NiIsIm9wZW5BY2Nlc3NJZCI6IjE1MTY5NDgyODU2MDM0NDAyNTI2In0sInRpdGxlIjoiVGhlIGFwcGxpY2F0aW9uIG9mIGFkdmFuY2VkIG94aWRhdGlvbiBwcm9jZXNzZXMgaW5jbHVkaW5nIHBob3RvY2F0YWx5c2lzLWJhc2VkIG9uZXMgZm9yIHRoZSBvZmYtZmxhdm91cnMgcmVtb3ZhbCAoR1NNIGFuZCBNSUIpIGluIHJlY2lyY3VsYXRpbmcgYXF1YWN1bHR1cmUgc3lzdGVtcyIsImFic3RyYWN0IjoiUmVjaXJjdWxhdGluZyBhcXVhY3VsdHVyZSBzeXN0ZW0gKFJBUykgcG9wdWxhcml0eSBpcyB0aHJlYXRlbmVkIGJ5IHRoZSBhY2N1bXVsYXRpb24gb2Ygb2ZmLWZsYXZvdXIgY29tcG91bmRzIHN1Y2ggYXMgZ2Vvc21pbiAoR1NNKSBhbmQgMi1tZXRoeWxpc29ib3JuZW9sIChNSUIpIGluIHRoZSByZWFyaW5nIHdhdGVyLiBEdWUgdG8gdGhlaXIgbGlwb3BoaWxpYyBuYXR1cmUgYW5kIHRoZSBoaWdoIGJpby1hY2N1bXVsYXRpb24gcmF0ZSwgdGhleSBjYW4gYmUgZGV0ZWN0ZWQgYXQgdmVyeSBsb3cgY29uY2VudHJhdGlvbnMgaW4gZmlzaCBmbGVzaCwgdGh1cyByZWR1Y2luZyBwcm9kdWN0IHF1YWxpdHkgYW5kIG1hcmtldGFiaWxpdHkuIFRoZSByZW1vdmFsIG9mIHRoZXNlIGNvbXBvdW5kcyBvZnRlbiByZXF1aXJlcyBmaXNoIHRvIGJlIHB1cmdlZCB1c2luZyBoaWdoIGFtb3VudHMgb2YgZnJlc2ggYW5kL29yIGhpZ2hseSB0cmVhdGVkIHdhdGVyIGJlZm9yZSBoYXJ2ZXN0aW5nLiBBIHByb21pc2luZyBhbHRlcm5hdGl2ZSBpcyBjb250cm9sbGluZyB0aGUgb2ZmLWZsYXZvdXJzIHRocm91Z2hvdXQgdGhlIHByb2R1Y3Rpb24gY3ljbGUgYnkgbWVhbnMgb2YgYWR2YW5jZWQgb3hpZGF0aW9uIHByb2Nlc3NlcyAoQU9QcykuIEluIHRoaXMgcmV2aWV3LCB0aGUgY3VycmVudCBzdGF0ZSBvZiB0aGUgYXJ0IG9mIEFPUHMgYXBwbGljYXRpb24gdG8gR1NNIGFuZCBNSUIgZGVncmFkYXRpb24gaXMgcmV2aWV3ZWQgdW5kZXIgdGhlIHBlcnNwZWN0aXZlIG9mIHRoZWlyIGFwcGxpY2FiaWxpdHkgdG8gUkFTcy4gSW4gcGFydGljdWxhciB0aGUgYWR2YW50YWdlcyBvZiBwcm9taXNpbmcgY291cGxlZCBwcm9jZXNzZXMgc3VjaCBhcyBwaG90b2NhdGFseXRpYyBvem9uYXRpb24gaGF2ZSBiZWVuIGhpZ2hsaWdodGVkLiBGaW5hbGx5LCB3ZSBjcml0aWNhbGx5IHJldmlld2VkIHRoZSBleGlzdGluZyBkZXRlY3Rpb24gbWV0aG9kcyBvZiBvZmYtZmxhdm91ciBjb21wb3VuZHMgaW4gdGhlIHJlYXJpbmcgd2F0ZXIsIGVtcGhhc2lzaW5nIHRob3NlIGFsbG93aW5nIHJhcGlkIGFuZCBjaGVhcCBtZWFzdXJlbWVudHMsIGluIG9yZGVyIHRvIGVuYWJsZSBSQVMgcGxhbnQgbWFuYWdlcnMgdG8gbWFrZSByZWFsLXRpbWUgaW5mb3JtZWQgZGVjaXNpb25zLCB3aGVuIG9mZi1mbGF2b3VycyBjb25jZW50cmF0aW9uIGluY3JlYXNlcyBhYm92ZSBhIGNlcnRhaW4gdGhyZXNob2xkLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2oubWNhdC4yMDIzLjExMzYxNiIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjEwMTYvai5tY2F0LjIwMjMuMTEzNjE2IiwiaXNzdWVkRGF0ZSI6IjIwMjMtMTAtMjYiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA4IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJNb2xlY3VsYXIgQ2F0YWx5c2lzIiwidm9sdW1lIjo1NTEsImltcGFjdEZhY3RvciI6IjQuNTkifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjoxMTM2MTYsImxhc3RQYWdlIjoxMTM2MTZ9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRWxzZXZpZXIgQlYifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjExMSwiY2l0ZWRCeUNvdW50IjoyMH19LCJxdW90ZSI6IlRoZSBub3QgIHZlcnkgaGlnaCBlZmZpY2llbmN5IG9mIHRoaXMgcHJvY2VzcyBpcyBjYXVzZWQgYnkgdGhlIHByZXNlbmNlIG9mIE5PTSwgIHdoaWNoIHNjYXZlbmdlcyB0aGUgaHlkcm94eWwgcmFkaWNhbHMgWzYxXSBhbmQgaXMgYSBsaW1pdGluZyBmYWN0b3Igb2YgdGhlICBlZmZpY2FjeSBvZiBWVVYgaXJyYWRpYXRpb24gaW4gUkFTIHN5c3RlbS4gUGxhc21hIFBsYXNtYSB0ZWNobmlxdWVzIHVzZSBhIG5vbi10aGVybWFsIHBsYXNtYSBkaXNjaGFyZ2UgdG8gZ2VuZXJhdGUgcmVhY3RpdmUgc3BlY2llcy4gSXRzIGFwcGxpY2F0aW9uIGZvciB0aGUgdHJlYXRtZW50IG9mIFJBUyByZWFyaW5nIHdhdGVyICBoYXMgbmV2ZXIgYmVlbiBkZW1vbnN0cmF0ZWQgYnV0IHRoZXJlIGlzIGEgd2lkZSByYW5nZSBvZiBhcHBsaWNhdGlvbiBpbiAgZm9vZCBpbmR1c3RyeSB3aGljaCBtYWtlIGl0IGEgcHJvbWlzaW5nIEFPUCBtZXRob2QgZm9yIHRoZSBkZWdyYWRhLXRpb24gb2Ygb3JnYW5pYyBjb21wb3VuZHMgWzM3LDYzLDY0XS4gTm9uLXRoZXJtYWwgcGxhc21hIChOVFApIGlzIGEgIHBsYXNtYSB3aG9zZSBjb25zdGl0dWVudHMgKG5vbi1pb25pemVkLCBpb25pemVkIHNwZWNpZXMsIGFuZCBmcmVlICBlbGVjdHJvbnMpIGFyZSBub3QgaW4gdGhlcm1hbCBlcXVpbGlicml1bS4gSXQgaXMgYWxzbyBjYWxsZWQgY29sZCBwbGFzbWEgIGJlY2F1c2UgaXRzIHRlbXBlcmF0dXJlIGlzIGNsb3NlIHRvIHRoZSBhbWJpZW50IG9uZS4gVmFyaW91cyBzb3VyY2VzIG9mICBjb2xkIHBsYXNtYSBhcmUgcG9zc2libGUsIHVzaW5nIGRpcmVjdCBjdXJyZW50LCBsb3cgZnJlcXVlbmN5IGFsdGVybmF0aW5nICBjdXJyZW50IChBQyksIHJhZGlvIGZyZXF1ZW5jeSBhbmQgbWljcm93YXZlIGRpc2NoYXJnZXMuIEFtb25nc3QgdGhlICBsb3cgZnJlcXVlbmN5IHRlY2huaXF1ZXMsIHRoZSBjb3JvbmEgZGlzY2hhcmdlIGFuZCB0aGUgZGllbGVjdHJpYyBiYXItcmllciBkaXNjaGFyZ2UgYXJlIHRoZSBtb3N0IGltcG9ydGFudCBvbmVzIFs2M10uICBUaGUgc2ltcGxlc3QgdHJlYXRtZW50IGNvbnNpc3RzIG9mIGEgZGlzY2hhcmdlIG9mIGdsb3dpbmcgY29sZCAgcGxhc21hIGluIHRoZSBhaXIsIGF0IGF0bW9zcGhlcmljIHByZXNzdXJlLCBqdXN0IGFib3ZlIHRoZSBmcmVlIHN1cmZhY2Ugb2YgIHdhdGVyLiBEaXNjaGFyZ2UgY2FuIGdlbmVyYXRlIG96b25lLCByZWFjdGl2ZSBveHlnZW4gYW5kIG5pdHJvZ2VuICBzcGVjaWVzLCBhbmQgT0ggcmFkaWNhbHMuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6NTAuMzcwNDQxNDM2NzY3NTgsImxlZnQiOjUxLjUwNDkwOTUxNTM4MDg2LCJ3aWR0aCI6NDIuNTcyNzc2Nzk0NDMzNTk0LCJoZWlnaHQiOjEuNzg1MTg5NjI4NjAxMDc0Mn19LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjUxLjgzNDQyMzA2NTE4NTU1LCJsZWZ0Ijo1MS41MDUxMzQ1ODI1MTk1Mywid2lkdGgiOjQyLjU2OTIwMjQyMzA5NTcsImhlaWdodCI6MS42NDI1NjAzNjI4MTU4NTd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo1My4xNDg2ODE2NDA2MjUsImxlZnQiOjUxLjUwNTEzNDU4MjUxOTUzLCJ3aWR0aCI6NDIuNTY2NjU4MDIwMDE5NTMsImhlaWdodCI6MS42NDI1NjAzNjI4MTU4NTd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo1NC40NzAwNjIyNTU4NTkzNzUsImxlZnQiOjUxLjUwNTEzNDU4MjUxOTUzLCJ3aWR0aCI6MjUuNjM5MTk0NDg4NTI1MzksImhlaWdodCI6MS42NDI1NjAzNjI4MTU4NTd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo1Ny45NDg2MDgzOTg0Mzc1LCJsZWZ0Ijo1MS41MDUxMzQ1ODI1MTk1Mywid2lkdGgiOjQuNDM0MzYyODg4MzM2MTgyLCJoZWlnaHQiOjEuNjQyNTYwMzYyODE1ODU3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6NjAuNTg0MzQ2NzcxMjQwMjM0LCJsZWZ0Ijo1My41MTQ1NDU0NDA2NzM4Mywid2lkdGgiOjQwLjU1ODQ2MDIzNTU5NTcsImhlaWdodCI6MS42NDI1NjAzNjI4MTU4NTd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo2MS44OTg2MDE1MzE5ODI0MiwibGVmdCI6NTEuNTA1MTM0NTgyNTE5NTMsIndpZHRoIjo0Mi41NjU4NTY5MzM1OTM3NSwiaGVpZ2h0IjoxLjY0MjU2MDM2MjgxNTg1N319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjYzLjIxOTk4OTc3NjYxMTMzLCJsZWZ0Ijo1MS41MDUxMzQ1ODI1MTk1Mywid2lkdGgiOjQyLjU2NTk5NDI2MjY5NTMxLCJoZWlnaHQiOjEuNjQyNTYwMzYyODE1ODU3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6NjQuNTM0MjU1OTgxNDQ1MzEsImxlZnQiOjUxLjUwNTEzNDU4MjUxOTUzLCJ3aWR0aCI6NDIuMTczOTA4MjMzNjQyNTgsImhlaWdodCI6MS42NDI1NjQxNzc1MTMxMjI2fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6NjUuODU1NzM1Nzc4ODA4NiwibGVmdCI6NTEuNTA1MTM0NTgyNTE5NTMsIndpZHRoIjo0Mi41Njg0MDEzMzY2Njk5MiwiaGVpZ2h0IjoxLjY0MjU2NDE3NzUxMzEyMjZ9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo2Ny4xNjk5OTA1Mzk1NTA3OCwibGVmdCI6NTEuNTA1MTM0NTgyNTE5NTMsIndpZHRoIjo0Mi41Njg1MzQ4NTEwNzQyMiwiaGVpZ2h0IjoxLjY0MjU2NDE3NzUxMzEyMjZ9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo2OC40OTEzNzExNTQ3ODUxNiwibGVmdCI6NTEuNTA1MTM0NTgyNTE5NTMsIndpZHRoIjo0Mi41NzE3NDMwMTE0NzQ2MSwiaGVpZ2h0IjoxLjY0MjU2NDE3NzUxMzEyMjZ9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo2OS44MDU2MjU5MTU1MjczNCwibGVmdCI6NTEuNTA1MTM0NTgyNTE5NTMsIndpZHRoIjo0Mi41NjU5OTQyNjI2OTUzMSwiaGVpZ2h0IjoxLjY0MjU2NDE3NzUxMzEyMjZ9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3MS4xMjcxMTMzNDIyODUxNiwibGVmdCI6NTEuNTA1MTM0NTgyNTE5NTMsIndpZHRoIjo0Mi41NzQwMjAzODU3NDIxOSwiaGVpZ2h0IjoxLjY0MjU2NDE3NzUxMzEyMjZ9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3Mi40NDEzNzU3MzI0MjE4OCwibGVmdCI6NTEuNTA1MTM0NTgyNTE5NTMsIndpZHRoIjo0Mi41NjY2NTgwMjAwMTk1MywiaGVpZ2h0IjoxLjY0MjU2NDE3NzUxMzEyMjZ9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3My43NjI3NzE2MDY0NDUzMSwibGVmdCI6NTEuNTA1MTM0NTgyNTE5NTMsIndpZHRoIjo0Mi4xNzU3ODUwNjQ2OTcyNjYsImhlaWdodCI6MS42NDI1NjQxNzc1MTMxMjI2fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6NzUuMDc3MDE4NzM3NzkyOTcsImxlZnQiOjUxLjUwNTEzNDU4MjUxOTUzLCJ3aWR0aCI6MjkuMTUwMDM1ODU4MTU0Mjk3LCJoZWlnaHQiOjEuNjQyNTY0MTc3NTEzMTIyNn19LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjc2LjM5ODM5OTM1MzAyNzM0LCJsZWZ0Ijo1My41MTQ1NDU0NDA2NzM4Mywid2lkdGgiOjQwLjU2MzU0OTA0MTc0ODA1LCJoZWlnaHQiOjEuNjQyNTY0MTc3NTEzMTIyNn19LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjc3LjcxMjc2MDkyNTI5Mjk3LCJsZWZ0Ijo1MS41MDUxMzQ1ODI1MTk1Mywid2lkdGgiOjQyLjU2NTk5NDI2MjY5NTMxLCJoZWlnaHQiOjEuNjQyNTY0MTc3NTEzMTIyNn19LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjc5LjAzNDE0MTU0MDUyNzM0LCJsZWZ0Ijo1MS41MDUxMzQ1ODI1MTk1Mywid2lkdGgiOjQyLjU3MjI4ODUxMzE4MzU5NCwiaGVpZ2h0IjoxLjY0MjU2NDE3NzUxMzEyMjZ9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo4MC4zNDg0MDM5MzA2NjQwNiwibGVmdCI6NTEuNTA1MTM0NTgyNTE5NTMsIndpZHRoIjo0Mi41NjY2NTgwMjAwMTk1MywiaGVpZ2h0IjoxLjY0MjU2NDE3NzUxMzEyMjZ9fV19LCJjaHVua0lkIjoiMTRhZjhhMGYtNjk2Ni00MWM0LWI4NTEtNjI3Njk4NTAzZTNiIiwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiMTEzNjIwIn19XQ%3D%3D. This transition is particularly relevant as plasma-activated water technology leverages the interaction between plasma and water to generate stable reactive species such as nitrate ions, nitrite ions, hydrogen peroxide, and peroxynitrite ions (Vikas & Kumar, 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IiIsImZhbWlseSI6IlZpa2FzIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlN1ZGhpciIsImZhbWlseSI6Ikt1bWFyIn19XSwiaWRzIjp7Imlzc24iOltdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMjEyMDMvcnMuMy5ycy0zNzg5ODM1L3YxIiwib3BlbmFsZXgiOiJXNDM5MDI3MDYwMiIsIm9wZW5BY2Nlc3NJZCI6IjcwMzkxMjc2NTA0MDU4MzYwMDIifSwidGl0bGUiOiJJbnZlc3RpZ2F0aW9uIHRoZSByb2xlIG9mIFBsYXNtYS1BY3RpdmF0ZWQgV2F0ZXIgV2FzaGluZyBhbmQgU3RvcmFnZSBvbiB0aGUgU2hlbGYgTGlmZSBvZiBDZXN0cnVtIG5vY3R1cm51bSBMLiAoTmlnaHQtQmxvb21pbmcgSmFzbWluZSkgRmxvd2VycyIsImFic3RyYWN0IjoiPHRpdGxlPkFic3RyYWN0PC90aXRsZT4gVGhpcyBzdHVkeSBpbnZlc3RpZ2F0ZXMgdGhlIGltcGFjdCBvZiBQbGFzbWEtQWN0aXZhdGVkIFdhdGVyIChQQVcpIHdhc2hpbmcgYW5kIHN0b3JhZ2Ugb24gdGhlIHNoZWxmIGxpZmUgb2YgTmlnaHQtQmxvb21pbmcgSmFzbWluZSAoTkJKKSBmbG93ZXJzICg8aXRhbGljPkNlc3RydW0gbm9jdHVybnVtPC9pdGFsaWM%2BIEwuKS4gTkJKIGZsb3dlcnMsIGtub3duIGZvciB0aGVpciBmcmFncmFudCBwcm9wZXJ0aWVzIGFuZCBkaXZlcnNlIGFwcGxpY2F0aW9ucywgaGF2ZSBhIHNob3J0IG5hdHVyYWwgc2hlbGYgbGlmZS4gUEFXIGlzIGdlbmVyYXRlZCB1c2luZyBhIHBlbmNpbCBwbGFzbWEgamV0IHdpdGggdmFyeWluZyBwbGFzbWEgdHJlYXRtZW50IHRpbWVzICgxMCB0byAzMCBtaW51dGVzKSBhbmQgaXMgdXRpbGl6ZWQgZm9yIHdhc2hpbmcgYW5kIHN0b3JpbmcgTkJKIGZsb3dlcnMuIFRoZSBmbG93ZXJzIHVuZGVyZ28gbWljcm9iaWFsIGFuYWx5c2lzLCBzZW5zb3J5IGV2YWx1YXRpb24sIHdlaWdodCBsb3NzIGFzc2Vzc21lbnQsIHNwb2lsYWdlIGV2YWx1YXRpb24sIGNvbG9yIGFuYWx5c2lzLCBhbmQgYW50aW94aWRhbnQgYWN0aXZpdHkgbWVhc3VyZW1lbnQgYWZ0ZXIgUEFXIHdhc2hpbmcgYW5kIHN0b3JhZ2UuIFBBVyBleGhpYml0cyBhbiBpbmNyZWFzaW5nIGFjaWRpdHkgYW5kIG94aWRhdGl2ZSBwb3RlbnRpYWwsIGFsb25nIHdpdGggYSBoaWdoZXIgY29uY2VudHJhdGlvbiBvZiByZWFjdGl2ZSBzcGVjaWVzIChOTzxzdWI%2BMzwvc3ViPsuJIGlvbnMsIE5PPHN1Yj4yPC9zdWI%2By4kgaW9ucywgSDxzdWI%2BMjwvc3ViPk88c3ViPjI8L3N1Yj4sIGRpc3NvbHZlZCBPPHN1Yj4zPC9zdWI%2BKSB3aXRoIGxvbmdlciBwbGFzbWEgdHJlYXRtZW50IHRpbWVzLiBXYXNoaW5nIE5CSiBmbG93ZXJzIHdpdGggUEFXIHNpZ25pZmljYW50bHkgcmVkdWNlcyBtaWNyb2JpYWwgZ3Jvd3RoLCB3ZWlnaHQgbG9zcywgYW5kIHNwb2lsYWdlLCBtYWludGFpbmluZyBiZXR0ZXIgc2Vuc29yeSBwcm9wZXJ0aWVzIGNvbXBhcmVkIHRvIGNvbnRyb2wtd2FzaGVkIGZsb3dlcnMuIEhvd2V2ZXIsIFBBVywgd2hlbiB1c2VkIGFzIGEgc3RvcmFnZSBtZWRpdW0sIG5lZ2F0aXZlbHkgaW1wYWN0cyB0aGUgZmxvd2VycywgY2F1c2luZyBveGlkYXRpdmUgZGFtYWdlLCBjb21wcm9taXNlZCBtZW1icmFuZSBpbnRlZ3JpdHksIGFuZCBoaWdoZXIgc3BvaWxhZ2UuIFBBVy13YXNoZWQgTkJKIGZsb3dlcnMgcmV0YWluIGNvbG9yIGludGVuc2l0eSBhbmQgZGVsYXkgYnJvd25pbmcgd2hpbGUgcHJlc2VydmluZyBhbnRpb3hpZGFudCBhY3Rpdml0eS4gUEFXLCBhcyBhIHdhc2hpbmcgbWVkaXVtLCBwcm92ZXMgZWZmZWN0aXZlIGluIHByZXNlcnZpbmcgdGhlIHF1YWxpdHkgb2YgZmxvd2Vycy4gQ2F1dGlvbiBpcyBhZHZpc2VkIHdoZW4gdXNpbmcgUEFXIGFzIGEgc3RvcmFnZSBtZWRpdW0sIGFzIHByb2xvbmdlZCBleHBvc3VyZSBsZWFkcyB0byBveGlkYXRpdmUgZGFtYWdlIGFuZCBjb21wcm9taXNlcyB0aGUgb3ZlcmFsbCBxdWFsaXR5IG9mIGZsb3dlcnMuIEZ1cnRoZXIgb3B0aW1pemF0aW9uIG9mIHBsYXNtYSB0cmVhdG1lbnQgdGltZXMgaXMgY3J1Y2lhbCBmb3IgYmFsYW5jaW5nIHRoZSBwb3NpdGl2ZSBhbmQgbmVnYXRpdmUgZWZmZWN0cyBvZiBQQVcgb24gZmxvd2VyIHByZXNlcnZhdGlvbi4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjEyMDMvcnMuMy5ycy0zNzg5ODM1L3YxIiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5yZXNlYXJjaHNxdWFyZS5jb20vYXJ0aWNsZS9ycy0zNzg5ODM1L2xhdGVzdC5wZGYiLCJpc3N1ZWREYXRlIjoiMjAyMy0xMi0yNyIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDciLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IlJlc2VhcmNoIFNxdWFyZSAoUmVzZWFyY2ggU3F1YXJlKSJ9LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJSZXNlYXJjaCBTcXVhcmUgKFVuaXRlZCBTdGF0ZXMpIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo1MywiY2l0ZWRCeUNvdW50IjowfX0sInF1b3RlIjoiUGxhc21hLWFjdGl2YXRlZCB3YXRlciAoUEFXKSB0ZWNobm9sb2d5IGhhcyBnYWluZWQgZ2xvYmFsIGF0dGVudGlvbiBpbiB0aGUgcGFzdCBkZWNhZGUgZHVlIHRvIGl0cyB2ZXJzYXRpbGUgYXBwbGljYXRpb25zIHNwYW5uaW5nIGhlYWx0aGNhcmUsIG1lZGljaW5lLCBmb29kIHByZXNlcnZhdGlvbiwgYW5kIGFncmljdWx0dXJlLCBldGMuKDE1LTM2KSBQbGFzbWEsIHJlY29nbml6ZWQgYXMgdGhlIGZvdXJ0aCBzdGF0ZSBvZiBtYXR0ZXIsIGludGVyYWN0cyB3aXRoIHdhdGVyLCBsZWFkaW5nIHRvIGNoYW5nZXMgaW4gaXRzIHBoeXNpY29jaGVtaWNhbCBwcm9wZXJ0aWVzIHRocm91Z2ggdGhlIGZvcm1hdGlvbiBhbmQgZGlzc29sdXRpb24gb2YgdmFyaW91cyByZWFjdGl2ZSBzcGVjaWVzICgzNy00MikuIFRoZSByZXN1bHRpbmcgd2F0ZXIsIG1vZGlmaWVkIGJ5IHBsYXNtYSBvbiBhIHBoeXNpb2NoZW1pY2FsIGxldmVsLCBpcyB0ZXJtZWQgcGxhc21hLWFjdGl2YXRlZCB3YXRlciAoUEFXKSAoMTUsIDQxLCA0My00NikuIFRoZSB0ZWNobm9sb2d5IGVuY29tcGFzc2luZyB0aGUgdXNlIG9mIFBBVywgaW5jbHVkaW5nIHBhdGhvZ2VuIGluYWN0aXZhdGlvbiAoZnVuZ2ksIHZpcnVzZXMsIGJhY3RlcmlhLCBwZXN0cywgZXRjLiksIGZvb2QgcHJlc2VydmF0aW9uLCBtZWRpY2luYWwgYXBwbGljYXRpb25zICh3b3VuZCBoZWFsaW5nLCBzZWxlY3RpdmUgY2FuY2VyIGNlbGwga2lsbGluZywgZXRjLiksIGFuZCBhZ3JpY3VsdHVyYWwgYmVuZWZpdHMgKGVuaGFuY2VkIGdlcm1pbmF0aW9uLCBudXRyaWVudCBwcm92aXNpb24gdG8gcGxhbnRzLCBldGMuKSwgaXMgY29sbGVjdGl2ZWx5IGtub3duIGFzIFBBVyB0ZWNobm9sb2d5ICgxNS0yMywgMjYsIDI3LCAzMCwgMzEsIDMzLCA0NCwgNDYtNDgpLiBUaGUgaW50ZXJhY3Rpb24gYmV0d2VlbiBwbGFzbWEgYW5kIHdhdGVyIGdlbmVyYXRlcyBzdGFibGUgcmVhY3RpdmUgc3BlY2llcyBzdWNoIGFzIG5pdHJhdGUgKE5P4oKDy4kpIGlvbnMsIG5pdHJpdGUgKE5P4oKCy4kpIGlvbnMsIGh5ZHJvZ2VuIHBlcm94aWRlIChI4oKCT%2BKCgiksIGRpc3NvbHZlZCBvem9uZSAoT%2BKCgyksIHBlcm94eW5pdHJpdGUgKE9OT0%2FLiSkgaW9ucywgZXRjLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjY0LjI4NDE0OTE2OTkyMTg4LCJsZWZ0IjoxMi4xMDkzNzY5MDczNDg2MzMsIndpZHRoIjo3Ni4yNDM3NTkxNTUyNzM0NCwiaGVpZ2h0IjoxLjI5MTUxOTI4NDI0ODM1Mn19LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjY3LjU2NjQxMzg3OTM5NDUzLCJsZWZ0IjoxMi4xMDkzNzY5MDczNDg2MzMsIndpZHRoIjo3Ni4zNDk4MTUzNjg2NTIzNCwiaGVpZ2h0IjoxLjI5MTUxOTI4NDI0ODM1Mn19LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjcwLjg0NTEwMDQwMjgzMjAzLCJsZWZ0IjoxMi4xMDkzNzY5MDczNDg2MzMsIndpZHRoIjo3Ni4yNDMwNDk2MjE1ODIwMywiaGVpZ2h0IjoxLjI5MTUxOTI4NDI0ODM1Mn19LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjc0LjEyNjE0NDQwOTE3OTY5LCJsZWZ0IjoxMi4xMDkzNzY5MDczNDg2MzMsIndpZHRoIjo3Ni4yMTA3MTYyNDc1NTg2LCJoZWlnaHQiOjEuMjkxNTE5Mjg0MjQ4MzUyfX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6NzcuNDA4NDE2NzQ4MDQ2ODgsImxlZnQiOjEyLjEwOTM3NjkwNzM0ODYzMywid2lkdGgiOjc2LjI5ODE0OTEwODg4NjcyLCJoZWlnaHQiOjEuMjkxNTE5Mjg0MjQ4MzUyfX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODAuNjg5NDY4MzgzNzg5MDYsImxlZnQiOjEyLjEwOTM3NjkwNzM0ODYzMywid2lkdGgiOjc2LjMzNzM3MTgyNjE3MTg4LCJoZWlnaHQiOjEuMjkxNTE5Mjg0MjQ4MzUyfX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODMuOTcxNzMzMDkzMjYxNzIsImxlZnQiOjEyLjEwOTM3NjkwNzM0ODYzMywid2lkdGgiOjc2LjIxNDg3NDI2NzU3ODEyLCJoZWlnaHQiOjEuMjkxNTE5Mjg0MjQ4MzUyfX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODcuMjQ5ODE2ODk0NTMxMjUsImxlZnQiOjEyLjEwOTM3NjkwNzM0ODYzMywid2lkdGgiOjc2LjIzMDQ2MTEyMDYwNTQ3LCJoZWlnaHQiOjEuMjkxNTE5Mjg0MjQ4MzUyfX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6OC45MzI1MjA4NjYzOTQwNDMsImxlZnQiOjEyLjEwOTM3NDA0NjMyNTY4NCwid2lkdGgiOjc2LjIwMDY5ODg1MjUzOTA2LCJoZWlnaHQiOjEuMjkxNTE3NDk2MTA5MDA4OH19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjEyLjIxNDc4ODQzNjg4OTY0OCwibGVmdCI6MTIuMTA5Mzc2OTA3MzQ4NjMzLCJ3aWR0aCI6NzYuMzM2NjU0NjYzMDg1OTQsImhlaWdodCI6MS4yOTE1MTc0OTYxMDkwMDg4fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6MTUuMjA2Mjk1MDEzNDI3NzM0LCJsZWZ0IjoxMi4xMDkzNzY5MDczNDg2MzMsIndpZHRoIjo3Ni4xODY0MjQyNTUzNzExLCJoZWlnaHQiOjEuNTc4NjQ4MzI4NzgxMTI4fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6MTguNDg4NTYxNjMwMjQ5MDIzLCJsZWZ0IjoxMi4xMDkzNzY5MDczNDg2MzMsIndpZHRoIjo3Ni4yMjMwODM0OTYwOTM3NSwiaGVpZ2h0IjoxLjU3ODY0ODMyODc4MTEyOH19XX0sImNodW5rSWQiOiI2Zjc5Y2VhMS1hZDUzLTRjOTYtOWE5NS1hZDZmZWE0MzJhZjEiLCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI1In19XQ%3D%3D.

Nanobubble Technology

2.1. Principles and Generation Methods

Nanobubbles are tiny gas-filled cavities, typically ranging from tens to hundreds of nanometers in diameter, distinguishing them from larger microbubbles or macrobubbles. Table 1 compares various nanobubble generation methods, highlighting their principles, advantages, and limitations. These nanoscale bubbles exhibit extraordinary physicochemical properties that set them apart in aqueous environments, making them highly relevant for applications in water treatment and aquaculture. Unlike conventional bubbles, which quickly rise to the surface due to buoyancy, nanobubbles demonstrate exceptional stability and can remain suspended for extended periods often days or even weeks. This longevity is attributed to their minuscule size, which minimizes buoyant forces governed by Stokes' law, where the terminal velocity of a bubble is proportional to the square of its radius. Consequently, nanobubbles remain dispersed throughout the water column, facilitating sustained interactions with dissolved substances and biological entities. Nanobubbles have a large surface-area-to-volume ratio orders of magnitude higher than that of larger bubbles, which enhances interfacial reactions and promotes efficient gas dissolution, mass transfer, and chemical reactivity. (Nghĩa et al., 2022)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5ndXnhu4VuIEjhu691IiwiZmFtaWx5IjoiTmdoxKlhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5ndXllbiBUaGkiLCJmYW1pbHkiOiJOZ3V5ZW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUGhhbiBUcuG7jW5nIiwiZmFtaWx5IjoiQsOsbmgifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTGUgVGhpIiwiZmFtaWx5IjoiTWF5In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRvbmcgVHJhbiIsImZhbWlseSI6Ikh1eSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQaOG6oW0gVGjDoWkiLCJmYW1pbHkiOiJHaWFuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTb3BoaWUiLCJmYW1pbHkiOiJTdOKAkEhpbGFpcmUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUGhhbiBUaGkiLCJmYW1pbHkiOiJWYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMjM0LTE3NDkiLCIyMjM0LTE3NTciXSwiaXNibiI6W10sImRvaSI6IjEwLjQ3ODUzL2Zhcy4yMDIyLmUzOSIsIm9wZW5hbGV4IjoiVzQyOTQ1OTc1NTQifSwidGl0bGUiOiJFZmZlY3Qgb2YgbmFub2J1YmJsZXMgKG94eWdlbiwgb3pvbmUpIG9uIHRoZSBQYWNpZmljIHdoaXRlIHNocmltcCAoUGVuYWV1cyB2YW5uYW1laSksIFZpYnJpbyBwYXJhaGFlbW9seXRpY3VzIGFuZCB3YXRlciBxdWFsaXR5IHVuZGVyIGxhYiBjb25kaXRpb25zIiwiYWJzdHJhY3QiOiJUaGlzIHN0dWR5IGFzc2Vzc2VkIHRoZSBlZmZlY3RzIG9mIG94eWdlbiBhbmQgb3pvbmUgbmFub2J1YmJsZXMgb24gZ2lsbCBtb3JwaG9sb2d5LCB3ZWlnaHQgZ2FpbiwgYW5kIG1vcnRhbGl0eSBvZiBQYWNpZmljIHdoaXRlIHNocmltcCAoUGVuYWV1cyB2YW5uYW1laSksIGFzIHdlbGwgYXMgdGhlIGxldmVsIG9mIFZpYnJpbyBwYXJhaGFlbW9seXRpY3VzIGFuZCB3YXRlciBxdWFsaXR5IG9mIHNocmltcCBjdWx0dXJlIHRhbmtzIHVuZGVyIGxhYiBjb25kaXRpb25zLiBUd28gZXhwZXJpbWVudHMgd2VyZSBjYXJyaWVkIG91dCB3aXRoIG94eWdlbiBtYWNyb2J1YmJsZSwgb3pvbmUgbWFjcm9idWJibGUsIG94eWdlbiBuYW5vYnViYmxlLCBvem9uZSBuYW5vYnViYmxlLCBhbmQgY29udHJvbCB0cmVhdG1lbnRzIChhaXItc3RvbmUgbWFjcm9idWJibGUpLiBFeHBlcmltZW50cyB3ZXJlIGRvbmUgaW4gdHJpcGxpY2F0ZSBpbiAxMDAgTCB0YW5rcyB3aXRoIDE14oCwIHNhbGluZSB3YXRlciwgYW5kIDIwIHNocmltcCBwZXIgdGFuay4gVGFua3MgaW4gRXhwZXJpbWVudCAxIHdlcmUgbm90IGlub2N1bGF0ZWQgd2l0aCBiYWN0ZXJpYTsgdGFua3MgaW4gRXhwZXJpbWVudCAyIHdlcmUgaW5vY3VsYXRlZCB3aXRoIFYuIHBhcmFoYWVtb2x5dGljdXMgYXQgYSBjb25jZW50cmF0aW9uIG9mIDEwNiBDRlUvbUwuIFRoZSByZXN1bHRzIHJldmVhbGVkIHRoYXQgc2hvcnQgdHJlYXRtZW50cyB3aXRoIG96b25lIG5hbm9idWJibGVzIGhhZCBtaW5pbWFsIGltcGFjdCBvbiBzaHJpbXAgZ2lsbHMsIG1vcnRhbGl0eSwgYW5kIGdyb3d0aCByYXRlcywgcmVkdWNlZCBWLiBwYXJhaGFlbW9seXRpY3VzIGNvbmNlbnRyYXRpb24gaW4gd2F0ZXIgY29tcGFyZWQgdG8gdGhlIG90aGVyIGdyb3VwcywgYW5kIGltcHJvdmUgd2F0ZXIgcXVhbGl0eS4gVGhlc2UgbGFib3JhdG9yeSByZXN1bHRzIGluZGljYXRlIHRoYXQgb3pvbmUgbmFub2J1YmJsZSB0cmVhdG1lbnQgbWF5IGJlIHVzZWZ1bCBmb3IgY29udHJvbGxpbmcgVi4gcGFyYWhhZW1vbHl0aWN1cy4gTW9yZSB3b3JrIGlzIG5lZWRlZCB0byBmaW5kIHRoZSBiZXN0IHByb3RvY29sIHRvIGFwcGx5IHRoZSB0ZWNobm9sb2d5IG9uIGEgY29tbWVyY2lhbCBzY2FsZS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuNDc4NTMvZmFzLjIwMjIuZTM5IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5lLWZhcy5vcmcvZG93bmxvYWQvZG93bmxvYWRfcGRmP3BpZD1mYXMtMjUtOC00MjkiLCJpc3N1ZWREYXRlIjoiMjAyMi0wOC0wMSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZpc2hlcmllcyBhbmQgYXF1YXRpYyBzY2llbmNlcyIsImlzc3VlIjo4LCJ2b2x1bWUiOjI1LCJpbXBhY3RGYWN0b3IiOiIxLjM0In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6NDI5LCJsYXN0UGFnZSI6NDQwfX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlNwcmluZ2VyIFNjaWVuY2UrQnVzaW5lc3MgTWVkaWEifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjMwLCJjaXRlZEJ5Q291bnQiOjEzfX0sInF1b3RlIjoiVGhlIGluZHVzdHJ5IHRoZXJlZm9yZSBuZWVkcyB0byBmaW5kIGFsdGVybmF0aXZlIG1lYXN1cmVzICB0byBwcmV2ZW50IGFuZCBjb250cm9sIHRoaXMgcGF0aG9nZW4uIE5hbm9idWJibGUgdGVjaG5vbG9neSBwcm9kdWNlcyBzbWFsbCBnYXMgYnViYmxlcyB3aXRoIGEgZGlhbWV0ZXIgb2YgbGVzcyB0aGFuIDIwMCBubSAoQWdyYXdhbCBldCBhbC4sIDIwMTE7IENoYXBsaW4sIDIwMjEpIHdoaWNoIGNhbiBleGlzdCBpbiB0aGUgd2F0ZXIgY29sdW1uIGZvciB3ZWVrcyAoQXpldmVkbyAgZXQgYWwuLCAyMDE2OyBQYXJtYXIgJiBNYWp1bWRlciwgMjAxMykuIFRoaXMgcGhlbm9tZW5vbiBpcyAgYXNzb2NpYXRlZCB3aXRoIHRoZSBwaHlzaWNhbCBjaGFyYWN0ZXJpc3RpYyBvZiB0aGVzZSBzbWFsbCBidWItYmxlcyAoV2VpanMgJiBMb2hzZSwgMjAxMykgYW5kIGhpZ2ggemV0YSBwb3RlbnRpYWwgdmFsdWUgd2hpY2ggIGtlZXBzIHRoZW0gZnJvbSBhZ2dyZWdhdGluZyBpbnRvIGxhcmdlciBidWJibGVzIChLaXJieSwgMjAxMDsgIFNqb2dyZWVuIGV0IGFsLiwgMjAxOCkuIE5hbm9idWJibGUgdGVjaG5vbG9neSBoYXMgYSBsb3Qgb2YgcHJvbS1pc2luZyBlZmZlY3RpdmUgYXBwbGljYXRpb24gaW4gYXF1YWN1bHR1cmUgYmVjYXVzZSBvZiBpdHMgYWJpbGl0eSAgdG8gaW5jcmVhc2UgYW5kIG1haW50YWluIG94eWdlbiBjb250ZW50IGluIHdhdGVyIGFuZCBjb250cm9sICBiYWN0ZXJpYSwgYSBwb3NzaWJsZSBtZWFzdXJlIHRvIHJlZHVjZSB0aGUgdXNlIG9mIGFudGliaW90aWNzLiBPeHlnZW4gbmFub2J1YmJsZXMgb3B0aW1pemUgdGhlIGRpc3NvbHZlZCBveHlnZW4gaW4gYXF1YXRpYyBzeXN0ZW1zIChHYWxhbmcgZXQgYWwuLCAyMDE5KS4gSGlnaCBsZXZlbHMgb2Ygb3h5Z2VuIG1heSBzcGVlZCB1cCBtZXRhYm9saXNtLCBwcm9tb3RlIGhpZ2hlciBmb29kIGludGFrZSwgYW5kIGluY3JlYXNlICBncm93dGggb2YgZmFybWVkIHNocmltcCAoRWJpbmEgZXQgYWwuLCAyMDEzKS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNS4wNjg2NDU0NzcyOTQ5MiwibGVmdCI6OC4wOTUyMzU4MjQ1ODQ5NjEsIndpZHRoIjo0MS4wNzIyMTIyMTkyMzgyOCwiaGVpZ2h0IjoxLjY5OTYzNTE0ODA0ODQwMDl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNi44MzI1MzA5NzUzNDE4LCJsZWZ0Ijo4LjA5NTIzNTgyNDU4NDk2MSwid2lkdGgiOjIzLjYwNDE4NTEwNDM3MDExNywiaGVpZ2h0IjoxLjY5OTYzNTE0ODA0ODQwMDl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC41OTY0MjAyODgwODU5NCwibGVmdCI6MTAuOTUyMzY0OTIxNTY5ODI0LCJ3aWR0aCI6MzguMjE1ODc3NTMyOTU4OTg0LCJoZWlnaHQiOjEuNjk5NjM1MTQ4MDQ4NDAwOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQwLjM2MDMwOTYwMDgzMDA4LCJsZWZ0Ijo4LjA5NTIzNTgyNDU4NDk2MSwid2lkdGgiOjQxLjA3Njc1OTMzODM3ODkwNiwiaGVpZ2h0IjoxNy41NzQ2MzA3MzczMDQ2ODh9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1Ny45OTkxOTUwOTg4NzY5NSwibGVmdCI6MTAuOTUyMzY0OTIxNTY5ODI0LCJ3aWR0aCI6MzguMjAxNDQ2NTMzMjAzMTI1LCJoZWlnaHQiOjEuNjk5NjM1MTQ4MDQ4NDAwOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjU5Ljc2MzA4MDU5NjkyMzgzLCJsZWZ0Ijo4LjA5NTIzNTgyNDU4NDk2MSwid2lkdGgiOjQxLjA2MzMyNzc4OTMwNjY0LCJoZWlnaHQiOjEuNjk5NjM1MTQ4MDQ4NDAwOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjYxLjUyNjk3MzcyNDM2NTIzNCwibGVmdCI6OC4wOTUyMzU4MjQ1ODQ5NjEsIndpZHRoIjo0MS4wNjY4NDQ5NDAxODU1NSwiaGVpZ2h0IjoxLjY5OTYzNTE0ODA0ODQwMDl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2My4yOTA4NjMwMzcxMDkzNzUsImxlZnQiOjguMDk1MjM1ODI0NTg0OTYxLCJ3aWR0aCI6NDAuNzE0MjkwNjE4ODk2NDg0LCJoZWlnaHQiOjEuNjk5NjMxMzMzMzUxMTM1M319XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjQzMCJ9fV0%3D.

Table 1: Comparison of nanobubble generation methods

| Method | Principle | Advantages | Disadvantages |
| --- | --- | --- | --- |
| Hydrodynamic cavitation | High-speed liquid flow through constricted areas | Simple, energy-efficient, scalable | Limited control over bubble size |
| Ultrasonic cavitation | Acoustic waves create cavitation bubbles | Precise control over bubble size | High energy consumption, limited scalability |
| Electrochemical generation | Electrolysis of water generates hydrogen nanobubbles | High purity, controllable bubble size | Complex setup, limited gas options |

A key factor contributing to nanobubble stability is their negatively charged surface, characterized by a high zeta potential. This electrostatic repulsion prevents coalescence, as like-charged bubbles repel each other, thereby preserving their diminutive size and preventing aggregation into larger, unstable forms. Research highlights that this zeta potential arises from the adsorption of hydroxide ions (OH⁻) at the gas-water interface, creating a robust electrical double layer that counters van der Waals attractions (Nghĩa et al., 2022)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5ndXnhu4VuIEjhu691IiwiZmFtaWx5IjoiTmdoxKlhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5ndXllbiBUaGkiLCJmYW1pbHkiOiJOZ3V5ZW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUGhhbiBUcuG7jW5nIiwiZmFtaWx5IjoiQsOsbmgifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTGUgVGhpIiwiZmFtaWx5IjoiTWF5In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRvbmcgVHJhbiIsImZhbWlseSI6Ikh1eSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQaOG6oW0gVGjDoWkiLCJmYW1pbHkiOiJHaWFuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTb3BoaWUiLCJmYW1pbHkiOiJTdOKAkEhpbGFpcmUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUGhhbiBUaGkiLCJmYW1pbHkiOiJWYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMjM0LTE3NDkiLCIyMjM0LTE3NTciXSwiaXNibiI6W10sImRvaSI6IjEwLjQ3ODUzL2Zhcy4yMDIyLmUzOSIsIm9wZW5hbGV4IjoiVzQyOTQ1OTc1NTQifSwidGl0bGUiOiJFZmZlY3Qgb2YgbmFub2J1YmJsZXMgKG94eWdlbiwgb3pvbmUpIG9uIHRoZSBQYWNpZmljIHdoaXRlIHNocmltcCAoUGVuYWV1cyB2YW5uYW1laSksIFZpYnJpbyBwYXJhaGFlbW9seXRpY3VzIGFuZCB3YXRlciBxdWFsaXR5IHVuZGVyIGxhYiBjb25kaXRpb25zIiwiYWJzdHJhY3QiOiJUaGlzIHN0dWR5IGFzc2Vzc2VkIHRoZSBlZmZlY3RzIG9mIG94eWdlbiBhbmQgb3pvbmUgbmFub2J1YmJsZXMgb24gZ2lsbCBtb3JwaG9sb2d5LCB3ZWlnaHQgZ2FpbiwgYW5kIG1vcnRhbGl0eSBvZiBQYWNpZmljIHdoaXRlIHNocmltcCAoUGVuYWV1cyB2YW5uYW1laSksIGFzIHdlbGwgYXMgdGhlIGxldmVsIG9mIFZpYnJpbyBwYXJhaGFlbW9seXRpY3VzIGFuZCB3YXRlciBxdWFsaXR5IG9mIHNocmltcCBjdWx0dXJlIHRhbmtzIHVuZGVyIGxhYiBjb25kaXRpb25zLiBUd28gZXhwZXJpbWVudHMgd2VyZSBjYXJyaWVkIG91dCB3aXRoIG94eWdlbiBtYWNyb2J1YmJsZSwgb3pvbmUgbWFjcm9idWJibGUsIG94eWdlbiBuYW5vYnViYmxlLCBvem9uZSBuYW5vYnViYmxlLCBhbmQgY29udHJvbCB0cmVhdG1lbnRzIChhaXItc3RvbmUgbWFjcm9idWJibGUpLiBFeHBlcmltZW50cyB3ZXJlIGRvbmUgaW4gdHJpcGxpY2F0ZSBpbiAxMDAgTCB0YW5rcyB3aXRoIDE14oCwIHNhbGluZSB3YXRlciwgYW5kIDIwIHNocmltcCBwZXIgdGFuay4gVGFua3MgaW4gRXhwZXJpbWVudCAxIHdlcmUgbm90IGlub2N1bGF0ZWQgd2l0aCBiYWN0ZXJpYTsgdGFua3MgaW4gRXhwZXJpbWVudCAyIHdlcmUgaW5vY3VsYXRlZCB3aXRoIFYuIHBhcmFoYWVtb2x5dGljdXMgYXQgYSBjb25jZW50cmF0aW9uIG9mIDEwNiBDRlUvbUwuIFRoZSByZXN1bHRzIHJldmVhbGVkIHRoYXQgc2hvcnQgdHJlYXRtZW50cyB3aXRoIG96b25lIG5hbm9idWJibGVzIGhhZCBtaW5pbWFsIGltcGFjdCBvbiBzaHJpbXAgZ2lsbHMsIG1vcnRhbGl0eSwgYW5kIGdyb3d0aCByYXRlcywgcmVkdWNlZCBWLiBwYXJhaGFlbW9seXRpY3VzIGNvbmNlbnRyYXRpb24gaW4gd2F0ZXIgY29tcGFyZWQgdG8gdGhlIG90aGVyIGdyb3VwcywgYW5kIGltcHJvdmUgd2F0ZXIgcXVhbGl0eS4gVGhlc2UgbGFib3JhdG9yeSByZXN1bHRzIGluZGljYXRlIHRoYXQgb3pvbmUgbmFub2J1YmJsZSB0cmVhdG1lbnQgbWF5IGJlIHVzZWZ1bCBmb3IgY29udHJvbGxpbmcgVi4gcGFyYWhhZW1vbHl0aWN1cy4gTW9yZSB3b3JrIGlzIG5lZWRlZCB0byBmaW5kIHRoZSBiZXN0IHByb3RvY29sIHRvIGFwcGx5IHRoZSB0ZWNobm9sb2d5IG9uIGEgY29tbWVyY2lhbCBzY2FsZS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuNDc4NTMvZmFzLjIwMjIuZTM5IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5lLWZhcy5vcmcvZG93bmxvYWQvZG93bmxvYWRfcGRmP3BpZD1mYXMtMjUtOC00MjkiLCJpc3N1ZWREYXRlIjoiMjAyMi0wOC0wMSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZpc2hlcmllcyBhbmQgYXF1YXRpYyBzY2llbmNlcyIsImlzc3VlIjo4LCJ2b2x1bWUiOjI1LCJpbXBhY3RGYWN0b3IiOiIxLjM0In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6NDI5LCJsYXN0UGFnZSI6NDQwfX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlNwcmluZ2VyIFNjaWVuY2UrQnVzaW5lc3MgTWVkaWEifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjMwLCJjaXRlZEJ5Q291bnQiOjEzfX0sInF1b3RlIjoiVGhlIGluZHVzdHJ5IHRoZXJlZm9yZSBuZWVkcyB0byBmaW5kIGFsdGVybmF0aXZlIG1lYXN1cmVzICB0byBwcmV2ZW50IGFuZCBjb250cm9sIHRoaXMgcGF0aG9nZW4uIE5hbm9idWJibGUgdGVjaG5vbG9neSBwcm9kdWNlcyBzbWFsbCBnYXMgYnViYmxlcyB3aXRoIGEgZGlhbWV0ZXIgb2YgbGVzcyB0aGFuIDIwMCBubSAoQWdyYXdhbCBldCBhbC4sIDIwMTE7IENoYXBsaW4sIDIwMjEpIHdoaWNoIGNhbiBleGlzdCBpbiB0aGUgd2F0ZXIgY29sdW1uIGZvciB3ZWVrcyAoQXpldmVkbyAgZXQgYWwuLCAyMDE2OyBQYXJtYXIgJiBNYWp1bWRlciwgMjAxMykuIFRoaXMgcGhlbm9tZW5vbiBpcyAgYXNzb2NpYXRlZCB3aXRoIHRoZSBwaHlzaWNhbCBjaGFyYWN0ZXJpc3RpYyBvZiB0aGVzZSBzbWFsbCBidWItYmxlcyAoV2VpanMgJiBMb2hzZSwgMjAxMykgYW5kIGhpZ2ggemV0YSBwb3RlbnRpYWwgdmFsdWUgd2hpY2ggIGtlZXBzIHRoZW0gZnJvbSBhZ2dyZWdhdGluZyBpbnRvIGxhcmdlciBidWJibGVzIChLaXJieSwgMjAxMDsgIFNqb2dyZWVuIGV0IGFsLiwgMjAxOCkuIE5hbm9idWJibGUgdGVjaG5vbG9neSBoYXMgYSBsb3Qgb2YgcHJvbS1pc2luZyBlZmZlY3RpdmUgYXBwbGljYXRpb24gaW4gYXF1YWN1bHR1cmUgYmVjYXVzZSBvZiBpdHMgYWJpbGl0eSAgdG8gaW5jcmVhc2UgYW5kIG1haW50YWluIG94eWdlbiBjb250ZW50IGluIHdhdGVyIGFuZCBjb250cm9sICBiYWN0ZXJpYSwgYSBwb3NzaWJsZSBtZWFzdXJlIHRvIHJlZHVjZSB0aGUgdXNlIG9mIGFudGliaW90aWNzLiBPeHlnZW4gbmFub2J1YmJsZXMgb3B0aW1pemUgdGhlIGRpc3NvbHZlZCBveHlnZW4gaW4gYXF1YXRpYyBzeXN0ZW1zIChHYWxhbmcgZXQgYWwuLCAyMDE5KS4gSGlnaCBsZXZlbHMgb2Ygb3h5Z2VuIG1heSBzcGVlZCB1cCBtZXRhYm9saXNtLCBwcm9tb3RlIGhpZ2hlciBmb29kIGludGFrZSwgYW5kIGluY3JlYXNlICBncm93dGggb2YgZmFybWVkIHNocmltcCAoRWJpbmEgZXQgYWwuLCAyMDEzKS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNS4wNjg2NDU0NzcyOTQ5MiwibGVmdCI6OC4wOTUyMzU4MjQ1ODQ5NjEsIndpZHRoIjo0MS4wNzIyMTIyMTkyMzgyOCwiaGVpZ2h0IjoxLjY5OTYzNTE0ODA0ODQwMDl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNi44MzI1MzA5NzUzNDE4LCJsZWZ0Ijo4LjA5NTIzNTgyNDU4NDk2MSwid2lkdGgiOjIzLjYwNDE4NTEwNDM3MDExNywiaGVpZ2h0IjoxLjY5OTYzNTE0ODA0ODQwMDl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC41OTY0MjAyODgwODU5NCwibGVmdCI6MTAuOTUyMzY0OTIxNTY5ODI0LCJ3aWR0aCI6MzguMjE1ODc3NTMyOTU4OTg0LCJoZWlnaHQiOjEuNjk5NjM1MTQ4MDQ4NDAwOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQwLjM2MDMwOTYwMDgzMDA4LCJsZWZ0Ijo4LjA5NTIzNTgyNDU4NDk2MSwid2lkdGgiOjQxLjA3Njc1OTMzODM3ODkwNiwiaGVpZ2h0IjoxNy41NzQ2MzA3MzczMDQ2ODh9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1Ny45OTkxOTUwOTg4NzY5NSwibGVmdCI6MTAuOTUyMzY0OTIxNTY5ODI0LCJ3aWR0aCI6MzguMjAxNDQ2NTMzMjAzMTI1LCJoZWlnaHQiOjEuNjk5NjM1MTQ4MDQ4NDAwOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjU5Ljc2MzA4MDU5NjkyMzgzLCJsZWZ0Ijo4LjA5NTIzNTgyNDU4NDk2MSwid2lkdGgiOjQxLjA2MzMyNzc4OTMwNjY0LCJoZWlnaHQiOjEuNjk5NjM1MTQ4MDQ4NDAwOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjYxLjUyNjk3MzcyNDM2NTIzNCwibGVmdCI6OC4wOTUyMzU4MjQ1ODQ5NjEsIndpZHRoIjo0MS4wNjY4NDQ5NDAxODU1NSwiaGVpZ2h0IjoxLjY5OTYzNTE0ODA0ODQwMDl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2My4yOTA4NjMwMzcxMDkzNzUsImxlZnQiOjguMDk1MjM1ODI0NTg0OTYxLCJ3aWR0aCI6NDAuNzE0MjkwNjE4ODk2NDg0LCJoZWlnaHQiOjEuNjk5NjMxMzMzMzUxMTM1M319XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjQzMCJ9fV0%3D. Nanobubbles also exhibit unique internal dynamics, such as high internal pressure (up to several hundred atmospheres) due to Laplace pressure, expressed as ΔP = 2σ/r, where σ is the surface tension and r is the bubble radius. This pressure stabilizes the bubbles against dissolution while enabling the generation of reactive oxygen species at the interface through localized cavitation or charge separation. These properties collectively enable nanobubbles to outperform traditional aeration methods in oxygen transfer efficiency and pollutant oxidation, crucial for maintaining optimal water quality in intensive aquaculture systems. Specifically, the elevated magnitude of zeta potential creates repulsion forces that prevent bubble coalescence, thereby contributing to the stabilization of the gas pockets within the liquid medium (Jia et al., 2023; Tekile et al., 2016)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjMwLCJzdWJ0eXBlIjp7fSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTWluZ3lpIiwiZmFtaWx5IjoiSmlhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik11aGFtbWFkIFVzbWFuIiwiZmFtaWx5IjoiRmFyaWQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSmVoYWQgQS4iLCJmYW1pbHkiOiJLaGFycmF6In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hbGxhcGFuZW5pIE1hbm9qIiwiZmFtaWx5IjoiS3VtYXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2hhdWhyYXQgUy4iLCJmYW1pbHkiOiJDaG9wcmEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW0iLCJmYW1pbHkiOiJKYW5nIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlkuTS4gSm9obiIsImZhbWlseSI6IkNoZXcifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2FtaXIgS3VtYXIiLCJmYW1pbHkiOiJLaGFuYWwifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiR3VhbmdoYW8iLCJmYW1pbHkiOiJDaGVuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFsaWNpYSBLeW91bmdqaW4iLCJmYW1pbHkiOiJBbiJ9fV0sImlkcyI6eyJpc3NuIjpbIjAwNDMtMTM1NCIsIjE4NzktMjQ0OCJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMTAxNi9qLndhdHJlcy4yMDIzLjEyMDYxMyIsInBtaWQiOiIzNzczODk0MCIsIm9wZW5hbGV4IjoiVzQzODY2MTg0NTkiLCJvcGVuQWNjZXNzSWQiOiIxMjc0NTg1MTk0MTU2NTI1MjY3MSJ9LCJ0aXRsZSI6Ik5hbm9idWJibGVzIGluIHdhdGVyIGFuZCB3YXN0ZXdhdGVyIHRyZWF0bWVudCBzeXN0ZW1zOiBTbWFsbCBidWJibGVzIG1ha2luZyBiaWcgZGlmZmVyZW5jZSIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2oud2F0cmVzLjIwMjMuMTIwNjEzIiwicmVzb3VyY2VVcmwiOiJodHRwczovL3B1cmVob3N0LmJhdGguYWMudWsvd3MvZmlsZXMvMjk2MzQzOTAxL05hbm9idWJibGVzX3ByZV9wcm9vZi5wZGYiLCJpc3N1ZWREYXRlIjoiMjAyMy0wOS0xMSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDgiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IldhdGVyIFJlc2VhcmNoIiwidm9sdW1lIjoyNDUsImltcGFjdEZhY3RvciI6IjEzLjQ0In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTIwNjEzLCJsYXN0UGFnZSI6MTIwNjEzfX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkVsc2V2aWVyIEJWIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjoxMTEsImNpdGVkQnlDb3VudCI6NzV9fSwicXVvdGUiOiJTZXZlcmFsIHN0dWRpZXMgaGF2ZSBwcm9wb3NlZCB0aGF0IHRoZSBuZWdhdGl2ZSBjaGFyZ2Ugb2YgTkJzIGFyaXNlcyBkdWUgdG8gdGhlaXIgc21hbGwgcmFkaXVzIGFuZCB0aGUgaGlnaCBwb2xhcml0eSBvZiBoeWRyb3h5bCBpb25zLCB3aGljaCBsZWFkIHRvIHRoZSBhZHNvcnB0aW9uIG9mIGRpc3NvY2lhdGVkIGh5ZHJveHlsIGlvbnMgb24gdGhlIGJ1YmJsZSBzdXJmYWNlLCB3aGlsZSBoeWRyb2dlbiBpb25zIGFyZSBlYXNpbHkgaHlkcmF0ZWQgYW5kIHByZWZlciB0byBzdGF5IGluIHRoZSBidWxrIGxpcXVpZC4gQSBoaWdoZXIgYWJzb2x1dGUgemV0YSBwb3RlbnRpYWwgaW5kaWNhdGVzIGhpZ2hlciBzdGFiaWxpdHksIGFzIHRoZSByZXB1bHNpdmUgZm9yY2UgZ2VuZXJhdGVkIGJ5IE5Cc%2BKAmCBzdXJmYWNlcyBwcmV2ZW50cyB0aGVtIGZyb20gbWVyZ2luZyB3aXRoIG9uZSBhbm90aGVyIChVc2hpa3VibyBldCBhbC4sIDIwMTApLiAxMSBTdXJmYWNlIGh5ZHJvcGhvYmljaXR5IGlzIGFub3RoZXIgZXNzZW50aWFsIHByb3BlcnR5IG9mIE5CcyBhcyBpdCBhbGxvd3MgTkJzIHRvIGludGVyYWN0IHdpdGggb3RoZXIgcGFydGljbGVzIHRocm91Z2ggaHlkcm9waG9iaWMgYXR0cmFjdGlvbi4gQk5CcyBjYW4gYWRzb3JiIG9uIGh5ZHJvcGhvYmljIHBhcnRpY2xlcyBhbmQgaW5jcmVhc2UgdGhlaXIgbW9iaWxpdHkgaW4gdGhlIGxpcXVpZCwgd2hpbGUgYXR0YWNoZWQgTkJzIGNhbiBwcm9tb3RlIHBhcnRpY2xlLXBhcnRpY2xlIG9yIHBhcnRpY2xlLSBidWJibGUgaW50ZXJhY3Rpb25zIHRocm91Z2ggY2FwaWxsYXJ5IGJyaWRnaW5nIGVmZmVjdHMgdGhhdCBwcm9kdWNlIGFuIGF0dHJhY3RpdmUgZm9yY2UgYmV0d2VlbiB0d28gc3VyZmFjZXMgKEhhbXB0b24gYW5kIE5ndXllbiwgMjAxMCkuIDIuMS4zIExvbmcgcmVzaWRlbmNlIHRpbWUgaW4gd2F0ZXI6IE5Cc%2BKAmSBsb25nLXRlcm0gc3RhYmlsaXR5IEFtb25nIHRoZSB2YXJpb3VzIGNoYXJhY3RlcmlzdGljcyBOQnMgcG9zc2VzcywgdGhlIG1vc3QgYXN0b25pc2hpbmcgaXMgdGhlaXIgbG9uZ2V2aXR5LiBVcG9uIGdlbmVyYXRpb24sIGJ1YmJsZXMgZ2VuZXJhbGx5IG1haW50YWluIHRoZWlyIGRpYW1ldGVycyBmb3IgYSBjZXJ0YWluIGFtb3VudCBvZiB0aW1lLCBhZnRlciB3aGljaCB0aGUgYnViYmxlcyBiZWdpbiB0byBncm93IGFuZCBldmVudHVhbGx5IGRpc3NvbHZlIGluIHRoZSBzb2x1dGlvbi4gRXhwZXJpbWVudGFsIHJlc3VsdHMgaGF2ZSBjb25maXJtZWQgdGhhdCBOQnMgYXJlIGFibGUgdG8gZXhpc3QgaW4gd2F0ZXIgZnJvbSBkYXlzIHRvIG1vbnRocy4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjExLCJiYm94Ijp7InRvcCI6NjguNjMxOTY1NjM3MjA3MDMsImxlZnQiOjkuNzQxMTgwNDE5OTIxODc1LCJ3aWR0aCI6NzcuMDEwMjUzOTA2MjUsImhlaWdodCI6MS44NDkyNjM2NjgwNjAzMDI3fX0seyJwYWdlTnVtYmVyIjoxMSwiYmJveCI6eyJ0b3AiOjcxLjgyMzgyMjAyMTQ4NDM4LCJsZWZ0Ijo5Ljc0MTE4MDQxOTkyMTg3NSwid2lkdGgiOjc3LjAwNjMwOTUwOTI3NzM0LCJoZWlnaHQiOjEuODQ5MjYzNjY4MDYwMzAyN319LHsicGFnZU51bWJlciI6MTEsImJib3giOnsidG9wIjo3NS4wMTIyMTQ2NjA2NDQ1MywibGVmdCI6OS43NDExODA0MTk5MjE4NzUsIndpZHRoIjo3Ni45NDg3NzYyNDUxMTcxOSwiaGVpZ2h0IjoxLjg0OTI2MzY2ODA2MDMwMjd9fSx7InBhZ2VOdW1iZXIiOjExLCJiYm94Ijp7InRvcCI6NzguMjAwNTk5NjcwNDEwMTYsImxlZnQiOjkuNzQxMTgwNDE5OTIxODc1LCJ3aWR0aCI6NzcuMDA4Mjg1NTIyNDYwOTQsImhlaWdodCI6MS44NDkyNjM2NjgwNjAzMDI3fX0seyJwYWdlTnVtYmVyIjoxMSwiYmJveCI6eyJ0b3AiOjgxLjM4ODk5OTkzODk2NDg0LCJsZWZ0Ijo5Ljc0MTE4MDQxOTkyMTg3NSwid2lkdGgiOjc3LjAxMDI0NjI3Njg1NTQ3LCJoZWlnaHQiOjEuODQ5MjYzNjY4MDYwMzAyN319LHsicGFnZU51bWJlciI6MTEsImJib3giOnsidG9wIjo4NC41NzY5MTk1NTU2NjQwNiwibGVmdCI6OS43NDExODA0MTk5MjE4NzUsIndpZHRoIjoxOS42MTY2MjQ4MzIxNTMzMiwiaGVpZ2h0IjoxLjg0OTI2MzY2ODA2MDMwMjd9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6ODcuNDg4MDU5OTk3NTU4NiwibGVmdCI6NDcuMDE4MjUzMzI2NDE2MDE2LCJ3aWR0aCI6Mi40NTIxNjE3ODg5NDA0Mjk3LCJoZWlnaHQiOjEuODQ5MjYzNjY4MDYwMzAyN319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoxMS42NDg3Mzg4NjEwODM5ODQsImxlZnQiOjkuNzQxMTgwNDE5OTIxODc1LCJ3aWR0aCI6NzYuOTUyNzA1MzgzMzAwNzgsImhlaWdodCI6MS44NDkyNjU1NzU0MDg5MzU1fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjE0LjgzOTQzNDYyMzcxODI2MiwibGVmdCI6OS43NDExODA0MTk5MjE4NzUsIndpZHRoIjo3Ni45NDg3ODM4NzQ1MTE3MiwiaGVpZ2h0IjoxLjg0OTI2NTU3NTQwODkzNTV9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MTguMDI3ODMzOTM4NTk4NjMzLCJsZWZ0Ijo5Ljc0MTE4MDQxOTkyMTg3NSwid2lkdGgiOjc2LjUwNjA3Mjk5ODA0Njg4LCJoZWlnaHQiOjEuODQ5MjY1NTc1NDA4OTM1NX19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyMS4yMTYyMTcwNDEwMTU2MjUsImxlZnQiOjkuNzQxMTgwNDE5OTIxODc1LCJ3aWR0aCI6NzYuOTg1NzMzMDMyMjI2NTYsImhlaWdodCI6MS44NDkyNjU1NzU0MDg5MzU1fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjI0LjQwNDYwMDE0MzQzMjYxNywibGVmdCI6OS43NDExODA0MTk5MjE4NzUsIndpZHRoIjozNS4zNzQxOTg5MTM1NzQyMiwiaGVpZ2h0IjoxLjg0OTI2NTU3NTQwODkzNTV9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MzIuMjExMzExMzQwMzMyMDMsImxlZnQiOjkuNzQxMTgwNDE5OTIxODc1LCJ3aWR0aCI6NTIuNDY0MTIyNzcyMjE2OCwiaGVpZ2h0IjoxLjkxOTk2NTI2NzE4MTM5NjV9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MzYuMDUyNjI3NTYzNDc2NTYsImxlZnQiOjkuNzQxMTgwNDE5OTIxODc1LCJ3aWR0aCI6NzcuMDA4Mjg1NTIyNDYwOTQsImhlaWdodCI6MS44NDkyNjc0ODI3NTc1Njg0fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM5LjI0MTAxNjM4NzkzOTQ1LCJsZWZ0Ijo5Ljc0MTE4MDQxOTkyMTg3NSwid2lkdGgiOjc3LjAwODI4NTUyMjQ2MDk0LCJoZWlnaHQiOjEuODQ5MjY3NDgyNzU3NTY4NH19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo0Mi40Mjk0MDkwMjcwOTk2MSwibGVmdCI6OS43NDExODA0MTk5MjE4NzUsIndpZHRoIjo3Ny4wMDI0MDMyNTkyNzczNCwiaGVpZ2h0IjoxLjg0OTI2NzQ4Mjc1NzU2ODR9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDUuNjE3Nzk0MDM2ODY1MjM0LCJsZWZ0Ijo5Ljc0MTE4MDQxOTkyMTg3NSwid2lkdGgiOjc3LjAxMDI0NjI3Njg1NTQ3LCJoZWlnaHQiOjEuODQ5MjY3NDgyNzU3NTY4NH19XX0sImNodW5rSWQiOiJlZWQ4MWMwZi01NzA0LTRmNjctYWRjNi1hMDk5YWNlNTEzZGUiLCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIxMjA2MjQifX0seyJvcmlnaW4iOjQsIndvcmsiOnsidHlwZSI6OCwic3VidHlwZSI6eyJvbmVvZktpbmQiOiJhcnRpY2xlVHlwZSIsImFydGljbGVUeXBlIjoxfSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW5kaW5ldCBLZWJlZGUiLCJmYW1pbHkiOiJUZWtpbGUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSWzigJBIbyIsImZhbWlseSI6IktpbSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJKYWl5ZW9wIiwiZmFtaWx5IjoiTGVlIn19XSwiaWRzIjp7Imlzc24iOlsiMTIyNi0xMDI1IiwiMjAwNS05NjhYIl0sImlzYm4iOltdLCJkb2kiOiIxMC40NDkxL2Vlci4yMDE2LjAyOCIsIm1hZyI6IjI1MjM2Njk2NTciLCJvcGVuYWxleCI6IlcyNTIzNjY5NjU3Iiwib3BlbkFjY2Vzc0lkIjoiMjg4OTU1Nzk5Mzc5ODk1NDAxMiJ9LCJ0aXRsZSI6IkV4dGVudCBhbmQgcGVyc2lzdGVuY2Ugb2YgZGlzc29sdmVkIG94eWdlbiBlbmhhbmNlbWVudCB1c2luZyBuYW5vYnViYmxlcyIsImFic3RyYWN0IjoiSW4gdGhpcyBzdHVkeSwgY2hhbmdlIGluIHdhdGVyLWRpc3NvbHZlZCBveHlnZW4gKERPKSB3YXMgYW5hbHl6ZWQgdW5kZXIgdmFyaW91cyBzeW50aGV0aWMgd2F0ZXIgcXVhbGl0aWVzIGFuZCBuYW5vYnViYmxlcyAoTkJzKSBhcHBsaWNhdGlvbiBjb25kaXRpb25zLCBzdWNoIGFzIGdhcyB0eXBlLCBpbml0aWFsIERPIGFzIHdlbGwgYXMgd2F0ZXIgZGlzc29sdmVkLCBzdXNwZW5kZWQgYW5kIG9yZ2FuaWMgbWF0dGVycyBjb250ZW50cy4gV2hlbiBveHlnZW4sIHJhdGhlciB0aGFuIGFpciwgd2FzIGludHJvZHVjZWQgaW50byBuaXRyb2dlbi1kZXNvcmJlZCB1bHRyYS1wdXJlIHdhdGVyLCB0aGUgc3RhZ25hdGlvbiB0aW1lIHdhcyBzaWduaWZpY2FudGx5IGluY3JlYXNlZC4gSXQgdG9vayB0ZW4gZGF5cyBmb3IgRE8gY29uY2VudHJhdGlvbiB0byBkcm9wIGJhY2sgdG8gc2F0dXJhdGlvbi4gVGhlIGhpZ2hlciB0aGUgaW5pdGlhbCBETyBjb25jZW50cmF0aW9uLCB0aGUgbG9uZ2VyIHBhcnRpY2xlcyB3ZXJlIG9ic2VydmVkIGFib3ZlIHNhdHVyYXRpb24gZHVlIHRvIHBhcnRpY2xlIHN0YWJpbGl0eSBpbXByb3ZlbWVudC4gVGhlIG94eWdlbiBtYXNzIHRyYW5zZmVyIHJhdGUgb2YgMC4wNDgyIG1nL0wvbWluIHdhcyBmb3VuZCB0byByZWFjaCBhIG1heGltdW0gYXQgYW4gZWxlY3Ryb2x5dGljIGNvbmNlbnRyYXRpb24gb2YgMC43NSBnL0wsIGJleW9uZCB3aGljaCB0aGUgdHJhbnNmZXIgcmF0ZSBkZWNyZWFzZWQgZHVlIHRvIGFkc29ycHRpb24gb2YgbmVnYXRpdmUgaW9ucyBvZiB0aGUgZWxlY3Ryb2x5dGUgYXQgdGhlIGludGVyZmFjZS4gSGlnaCBsZXZlbHMgb2YgdHVyYmlkaXR5IGNhdXNlZCBieSBzdXNwZW5kZWQgc29saWRzIGhhdmUgYmVjb21lIGEgYmFycmllciB0byBkaXNzb2x1dGlvbiBvZiBOQnMgb3h5Z2VuIGludG8gdGhlIHdhdGVyIHNvbHV0aW9uLCBhbmQgdGh1cyBhZmZlY3RlZCB0aGUgdHJhbnNmZXIgcGVyZm9ybWFuY2UuIE9uIHRoZSBvdGhlciBoYW5kLCBieSBhcHBseWluZyBOQnMgZm9yIGp1c3QgYW4gaG91ciwgdXAgdG8gNy4yJSBkZWdyYWRhdGlvbiBvZiBnbHVjb3NlIGFzIHJlcHJlc2VudGF0aXZlIG9yZ2FuaWMgbWF0dGVyIHdhcyBhY2hpZXZlZC4gVGh1cywgTkJzIHRlY2hub2xvZ3kgd291bGQgbWFpbnRhaW4gYSBoaWdoIERPIGV4dGVudCBmb3IgYW4gZXh0ZW5kZWQgZHVyYXRpb24sIGFuZCB0aHVzIGNhbiBpbXByb3ZlIHdhdGVyIHF1YWxpdHkgcHJvdmlkZWQgdGhhdCB3YXRlciBjaGVtaXN0cnkgaXMgY2xvc2VseSBtb25pdG9yZWQgZHVyaW5nIGl0cyBhcHBsaWNhdGlvbi4gS2V5d29yZHM6IEFwcGxpY2F0aW9uIGNvbmRpdGlvbnMsIERpc3NvbHZlZCBveHlnZW4sIE5hbm9idWJibGVzLCBXYXRlciBxdWFsaXR5IiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjQ0OTEvZWVyLjIwMTYuMDI4IiwicmVzb3VyY2VVcmwiOiJodHRwOi8vZWVlci5vcmcvdXBsb2FkL2Vlci0yMS00LTQyNy5wZGYiLCJpc3N1ZWREYXRlIjoiMjAxNi0wOS0xMiIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkVudmlyb25tZW50YWwgRW5naW5lZXJpbmcgUmVzZWFyY2giLCJpc3N1ZSI6NCwidm9sdW1lIjoyMSwiaW1wYWN0RmFjdG9yIjoiNC4wNSJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjQyNywibGFzdFBhZ2UiOjQzNX19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJLb3JlYW4gU29jaWV0eSBvZiBFbnZpcm9ubWVudGFsIEVuZ2luZWVyaW5nIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjoyNCwiY2l0ZWRCeUNvdW50IjozM319LCJxdW90ZSI6IlRoaXMgdmVyaWZpZXMgdGhlIHBvc3NpYmlsaXR5IG9mIHRoZSBleGlzdGVuY2UgIG9mIE5Ccywgc2luY2UgYXQgYSBoaWdoZXIgRE8gY29uY2VudHJhdGlvbiwgdGhlIG94eWdlbiBkaWZmdXNpb24gIHdvdWxkIHRha2UgbW9yZSB0aW1lLiBUaGUgaGlnaCBnYXMgY29uY2VudHJhdGlvbiBpbiB0aGUgbGlxdWlkICBleHBsYWlucyB0aGUgZXh0ZW5zaW9uIG9mIE5CIHN0YWJpbGl0eS4gIFRoZSBtYWduaXR1ZGVzIG9mIHpldGEgcG90ZW50aWFsIG9mIG94eWdlbiBuYW5vLWJ1YmJsZWQgd2F0ZXIgIHdlcmUgcG9zc2libHkgaGlnaGVyIHRoYW4gdGhvc2Ugb2YgYWlyIG5hbm8tYnViYmxlZCB3YXRlci4gQXQgIGEgaGlnaCBtYWduaXR1ZGUgemV0YSBwb3RlbnRpYWwgdmFsdWUsIHRoZSBlbGVjdHJpY2FsbHkgY2hhcmdlZCAgcGFydGljbGVzIHRlbmQgdG8gcmVwZWwgZWFjaCBvdGhlciwgYXZvaWRpbmcgYWdncmVnYXRpb24uIEZvciBidWJibGUgIGRpc3BlcnNpb24sIHRoZSBoaWdoIHpldGEgcG90ZW50aWFsIGNvdWxkIGNyZWF0ZSByZXB1bHNpb24gZm9yY2VzICB0aGF0IHdvdWxkIGF2b2lkIHRoZSBjb2FsZXNjZW5jZSBvZiBidWJibGVzLCB0aHVzIGNvbnRyaWJ1dGluZyAgdG8gdGhlIHN0YWJpbGl6YXRpb24gb2YgdGhlIGJ1YmJsZXMuIFRoZXJlZm9yZSwgdGhlIG94eWdlbiBOQnMgIGNvdWxkIGJlIHN0YWJpbGl6ZWQgaW4gd2F0ZXIgYnkgdGhlIGVsZWN0cmljYWxseSBjaGFyZ2VkIHN1cmZhY2VzLCAgd2hpbGUgdGhlIG5hbm8tYnViYmxlZCBhaXIgd291bGQgYmUgYmVsb3cgdGhlIGxpbWl0IG9mIHRoZSBzdGFiaWxpemEtdGlvbiBieSByZXB1bHNpb24gZm9yY2VzLiBGdXJ0aGVybW9yZSwgdGhlIGJpZyBkaWZmZXJlbmNlIGJldHdlZW4gdGhlIHBlcmlvZHMgaW4gd2hpY2ggIGJ1YmJsZXMgY291bGQgYmUgb2JzZXJ2ZWQgYXMgc3RhYmxlIGluIHRoZSBveHlnZW4gbmFuby1idWJibGVkICB3YXRlciAoMTAtMTUgZCkgYW5kIGluIHRoYXQgb2YgYWlyIG5hbm8tYnViYmxlZCAoMy01IGQpIGlzIGxpa2VseSBGaWcuIDYuIEdyYXBocyBzaG93aW5nIHN0YWduYXRpb24gdGltZSBmb3IgZGlmZmVyZW50IGluaXRpYWwgZGlzc29sdmVkIG94eWdlbiAoRE8pLiByZWxhdGVkIHRvIHRoZSBsb3dlciBkaXNzb2x1dGlvbiBvZiB0aGUgYWlyIGluIHdhdGVyIGluIHJlbGF0aW9uIHRvIHRoZSBkaXNzb2x1dGlvbiBvZiBveHlnZW4uICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6NjYuMTc4ODAyNDkwMjM0MzgsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6NDAuNzE0ODg5NTI2MzY3MTksImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo2Ny41NzY3NzQ1OTcxNjc5NywibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42ODgyNzA1Njg4NDc2NTYsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo2OC45NjcyMzE3NTA0ODgyOCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NTcxODA3ODYxMzI4MSwiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjcwLjM1NzY4ODkwMzgwODYsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6MjUuMzQxMjU1MTg3OTg4MjgsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3MS43NTU2NTMzODEzNDc2NiwibGVmdCI6OS41NDk2NDE2MDkxOTE4OTUsIndpZHRoIjozOS4wMjQ3ODQwODgxMzQ3NjYsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3My4xNDYxMDI5MDUyNzM0NCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NTQwMjk4NDYxOTE0MDYsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3NC41MzY1NjAwNTg1OTM3NSwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NTM0NjUyNzA5OTYwOTQsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3NS45MzQ1MzIxNjU1MjczNCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NzMyMzY4NDY5MjM4MywiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjc3LjMyNDk4OTMxODg0NzY2LCJsZWZ0Ijo3Ljg2NTU0NTI3MjgyNzE0OCwid2lkdGgiOjQwLjY0OTM1MzAyNzM0Mzc1LCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6NzguNzE1NDQ2NDcyMTY3OTcsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6NDAuNjU0NjI0OTM4OTY0ODQ0LCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODAuMTEzNDEwOTQ5NzA3MDMsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6NDAuNjUxMDUwNTY3NjI2OTUsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo4MS41MDM4NjA0NzM2MzI4MSwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NTQzNDI2NTEzNjcxOSwiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjgyLjg5NDMxNzYyNjk1MzEyLCJsZWZ0Ijo3Ljg2NTU0NTI3MjgyNzE0OCwid2lkdGgiOjQwLjMyMTgyMzEyMDExNzE5LCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODQuMjkyMjg5NzMzODg2NzIsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6MTUuODcwMjk1NTI0NTk3MTY4LCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODUuNjgyNzQ2ODg3MjA3MDMsImxlZnQiOjkuNTQ5NjQxNjA5MTkxODk1LCJ3aWR0aCI6MzguOTg5NzA0MTMyMDgwMDgsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo4Ny4wNzMyMDQwNDA1MjczNCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC43MTg0OTQ0MTUyODMyLCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODguNDcxMTY4NTE4MDY2NCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42ODc0MTk4OTEzNTc0MiwiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjM0LjUxMzI3MTMzMTc4NzExLCJsZWZ0Ijo1MS45MzI3Nzc0MDQ3ODUxNTYsIndpZHRoIjo0MC42NDcwNTI3NjQ4OTI1OCwiaGVpZ2h0IjoyLjcxODY2NjU1MzQ5NzMxNDV9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjozOC43MjUxNTg2OTE0MDYyNSwibGVmdCI6NTEuOTMyNzc3NDA0Nzg1MTU2LCJ3aWR0aCI6NDAuNjQzMTM1MDcwODAwNzgsImhlaWdodCI6MS4yNTMyODg1MDc0NjE1NDc5fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6NDAuMTE1NjEyMDMwMDI5MywibGVmdCI6NTEuOTMyNzc3NDA0Nzg1MTU2LCJ3aWR0aCI6NDAuNjY0NTg1MTEzNTI1MzksImhlaWdodCI6MS4yNTMyODg1MDc0NjE1NDc5fX1dfSwiY2h1bmtJZCI6IjFiMDNhYzE5LWZmZmItNDBkYy04N2QyLTRiM2QzOTEyODQxZiIsImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjQzMSJ9fV0%3D. The thermodynamic metastability of these charged cavities is further reinforced by charge enrichment at the interface, which creates an electric field energy that establishes a local minimum in the free-energy cost of bubble production (Sakr et al., 2025)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1hcndhIiwiZmFtaWx5IjoiU2FrciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNb2hhbWVkIE0uIiwiZmFtaWx5IjoiTW9oYW1lZCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNb2hhbWVkIElicmFoaW0iLCJmYW1pbHkiOiJLaXpoaXNzZXJpIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik11bmplZCBBLiIsImZhbWlseSI6Ik1hcmFxYSJ9fV0sImlkcyI6eyJpc3NuIjpbIjIwNDUtMjMyMiJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMTAzOC9zNDE1OTgtMDI1LTIxMjkzLXkiLCJwbWlkIjoiNDExMzY2MTQiLCJvcGVuYWxleCI6Ilc0NDE1NTAyNTY4Iiwib3BlbkFjY2Vzc0lkIjoiMTY1NjA5NTkyMDE3NTIwNzQ0OTAifSwidGl0bGUiOiJDaGFyYWN0ZXJpemF0aW9uIG9mIGFpciBzdWJtaWNyb2J1YmJsZXMgZm9yIHdhdGVyIHRyZWF0bWVudCB1bmRlciBkaWZmZXJlbnQgZ2VuZXJhdGlvbiBjb25kaXRpb25zIiwiYWJzdHJhY3QiOiJNaWNyby1uYW5vYnViYmxlIChNTkIpIHRlY2hub2xvZ3kgaGFzIGJlZW4gcHJvdmVuIHRvIGJlIGVmZmVjdGl2ZSBpbiB3YXRlciBhbmQgd2FzdGV3YXRlciB0cmVhdG1lbnQuIFN1Ym1pY3JvYnViYmxlcyAoU01CcykgYXJlIGNvbnNpZGVyZWQgdG8gYmUgYSBzdWJncm91cCBvZiBNTkJzIHJhbmdpbmcgZnJvbSAxIHRvIDEwIM68bSB3aXRoIGxpbWl0ZWQgc3R1ZGllcyByZWxhdGVkIHRvIHRoZWlyIGZ1bmRhbWVudGFsIHByb3BlcnRpZXMuIFRoaXMgc3R1ZHkgZm9jdXNlZCBvbiB0aGUgZnVuZGFtZW50YWwgY2hhcmFjdGVyaXN0aWNzIG9mIFNNQnMgYW5kIHRoZSBlZmZlY3Qgb2YgZ2VuZXJhdGlvbiBjb25kaXRpb25zIHN1Y2ggYXMgdGVtcGVyYXR1cmUsIGFlcmF0aW9uIHRpbWUsIGFuZCB3YXRlciB2b2x1bWUgb24gdGhlaXIgcHJvcGVydGllcy4gU01CcyB3ZXJlIGdlbmVyYXRlZCB1bmRlciBoaWdoIHByZXNzdXJlIHVzaW5nIHNoZWFyIGZvcmNlIGFuZCB0aGVpciBzaXplIGFuZCBkaXN0cmlidXRpb24gd2VyZSBtZWFzdXJlZCB1c2luZyB0aGUgZHluYW1pYyBsaWdodCBzY2F0dGVyaW5nIG1ldGhvZC4gRGlzc29sdmVkIG94eWdlbiAoRE8pIGNvbmNlbnRyYXRpb25zIHdlcmUgbW9uaXRvcmVkIGR1cmluZyBhbmQgYWZ0ZXIgdGhlIGdlbmVyYXRpb24uIFRoZSB6ZXRhIHBvdGVudGlhbCBvZiB0aGUgZ2VuZXJhdGVkIGJ1YmJsZXMgd2FzIGFsc28gbWVhc3VyZWQgdG8gYXNzZXNzIGJ1YmJsZXMgc3RhYmlsaXR5LiBTTUJzIHdpdGggYSBtZWRpYW4gZGlhbWV0ZXIgb2YgMiDOvG0gcGVyc2lzdGVkIGluIHdhdGVyIGV2ZW4gYWZ0ZXIgdGhlIGdlbmVyYXRvciB3YXMgc3RvcHBlZCwgaW5kaWNhdGluZyB0aGUgaGlnaCBsb25nZXZpdHkgb2YgU01CcyBpbiB3YXRlci4gUmVnYXJkbGVzcyBvZiB0aGUgYWVyYXRpb24gdGltZSBvciB3YXRlciB2b2x1bWUgdXNlZCwgdGhlIHpldGEgcG90ZW50aWFsIG9mIFNNQnMgd2FzIGhpZ2hseSBuZWdhdGl2ZSB3aXRoIGF2ZXJhZ2UgdmFsdWVzIHJhbmdlZCBiZXR3ZWVuIOKIkiAyOCBhbmQg4oiSIDMwIG1WLCBpbmRpY2F0aW5nIGhpZ2ggc3RhYmlsaXR5IGluIHdhdGVyLiBETyBjb25jZW50cmF0aW9uIGluY3JlYXNlZCBieSB1cCB0byAxLjUgZm9sZHMgd2l0aGluIGEgZmV3IG1pbnV0ZXMgb2YgZ2VuZXJhdGlvbiBhbmQgc2xpZ2h0bHkgZGVjcmVhc2VkIG92ZXIgMSBoLiBSZXN1bHRzIGRlbW9uc3RyYXRlIHRoYXQgYWlyIFNNQnMgYXJlIHN0YWJsZSB3aXRoIGxvbmcgbGlmZXNwYW4gYW5kIGhpZ2ggRE8gY29uY2VudHJhdGlvbi4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAzOC9zNDE1OTgtMDI1LTIxMjkzLXkiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vd3d3Lm5hdHVyZS5jb20vYXJ0aWNsZXMvczQxNTk4LTAyNS0yMTI5My15LnBkZiIsImlzc3VlZERhdGUiOiIyMDI1LTEwLTI0IiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiU2NpZW50aWZpYyBSZXBvcnRzIiwiaXNzdWUiOjEsInZvbHVtZSI6MTUsImltcGFjdEZhY3RvciI6IjQuOTEifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiTmF0dXJlIFBvcnRmb2xpbyJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NjQsImNpdGVkQnlDb3VudCI6MH19LCJxdW90ZSI6IlRoZSB2YWx1ZXMgY291bGQgZGlmZmVyIGZyb20gb25lIGV4cGVyaW1lbnQgdG8gYW5vdGhlciwgZGVwZW5kaW5nIG9uIHRoZSBvdmVyYWxsIGV4cGVyaW1lbnRhbCBjb25kaXRpb25zLiBUaGlzIGlzIGNvbW1vbiBhbW9uZyBhbGwgZXhwZXJpbWVudHMgaW4gd2hpY2ggY2hhcmdlZCAgc3VyZmFjZXMgYXJlIHByZXNlbnQsIHdoZXJlYXMgdGhlIGFic29sdXRlIHZhbHVlIGRlcGVuZHMgbW9yZSBvbiB0aGUgZXhwZXJpbWVudGFsIGNvbmRpdGlvbnMuIFJlZ2FyZGxlc3Mgb2YgIHRoZSBhYnNvbHV0ZSB2YWx1ZSBvZiDOtiwgdGhlIGNoYXJnZWQgTU5CcyB3aWxsIGJlIHJlcGVsbGluZyBlYWNoIG90aGVyLCB0aHVzIHByZXZlbnRpbmcgYnViYmxlIGNvYWxlc2NlbmNlLiAgVGhpcyBoZWxwcyB0byByZXRhaW4gYnViYmxlcyBpbiB0aGUgc29sdXRpb24gZm9yIGEgbG9uZ2VyIHRpbWUuIE5lZ2F0aXZlbHkgY2hhcmdlZCBidWJibGVzLCB3aXRoIHRoZWlyIGxvbmdldml0eSAgaW4gd2F0ZXIsIGFzc2lzdCBpbiB0aGUgdHJlYXRtZW50IHByb2Nlc3MuIEluIGFkZGl0aW9uLCBoaWdoIERPIGNvbnRlbnRzIGluIHdhdGVyIGZhY2lsaXRhdGUgcHJvY2Vzc2VzIHN1Y2ggYXMgRmlnLiA4LiAoY29udGludWVkKSBGaWcuIDkuIERPIGNvbmNlbnRyYXRpb24gb2YgU01CcyBhdCBkaWZmZXJlbnQgYWVyYXRpb24gdGltZXMgYW5kIHZvbHVtZXMgYW5kIHRoZWlyIGxvbmdldml0eSBhZnRlciBnZW5lcmF0b3Igc3RvcHBhZ2UuIGFlcmF0aW9uLCB3aGljaCByZXF1aXJlIGEgaGlnaCBveHlnZW4gY29udGVudC4gQWNjb3JkaW5nbHksIHRoZSB1c2Ugb2YgTUJzL1NNQnMvTkJzIGlzIG1vcmUgYmVuZWZpY2lhbCB0aGFuIGNvbnZlbnRpb25hbCBwcm9jZXNzZXMuIFRoZSBwaHlzaWNhbCBtZWNoYW5pc20gb2YgYnVsayBOQiBzdGFiaWxpdHkgaGFzIGJlZW4gZXhwbGFpbmVkIGJ5IFpoYW5nIGV0IGFsLiAoMjAyMCk2Ni4gQ2hhcmdlIGVucmljaG1lbnQgaXMgY2F1c2VkIGJ5IHRoZSBzaWduaWZpY2FudCBhZmZpbml0eSBvZiBuZWdhdGl2ZSBjaGFyZ2VzIGZvciB0aGUgTkIgaW50ZXJmYWNlLCB3aGljaCByZXN1bHRzIGluIGVsZWN0cmljIGZpZWxkIGVuZXJneSB0aGF0IGdpdmVzIHJpc2UgdG8gYSBsb2NhbCBtaW5pbXVtIGluIHRoZSBmcmVlLWVuZXJneSBjb3N0IG9mIGJ1YmJsZSBwcm9kdWN0aW9uIGFuZCB0aGVybW9keW5hbWljIG1ldGFzdGFiaWxpdHkgIG9mIHRoZSBjaGFyZ2VkIE5Ccy4gQSBzaXplLWRlcGVuZGVudCBmb3JjZSBpcyBnZW5lcmF0ZWQgbWVjaGFuaWNhbGx5IGJ5IGV4Y2VzcyBzdXJmYWNlIGNoYXJnZXMuIFRoaXMgZm9yY2Ugc2VydmVzICBhcyBhIHJlc3RvcmluZyBmb3JjZSB3aGVuIHRoZSBOQiBpcyB0aGVybW9keW5hbWljYWxseSBkaXN0dXJiZWQgZnJvbSBpdHMgZXF1aWxpYnJpdW0gc3RhdGUgYW5kIGJhbGFuY2VzIHRoZSAgTGFwbGFjZSBwcmVzc3VyZS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6ODUuNzA5ODYxNzU1MzcxMSwibGVmdCI6MjYuMTkwNDU2MzkwMzgwODYsIndpZHRoIjo2Ny4yNTUwNjU5MTc5Njg3NSwiaGVpZ2h0IjoxLjU1MTgzODA0MDM1MTg2Nzd9fSx7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6ODYuOTg4MDQ0NzM4NzY5NTMsImxlZnQiOjI2LjE5MDQ1NjM5MDM4MDg2LCJ3aWR0aCI6NjcuMjU0MzEwNjA3OTEwMTYsImhlaWdodCI6MS41NTE4MzgwNDAzNTE4Njc3fX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjg4LjI2NjIyMDA5Mjc3MzQ0LCJsZWZ0IjoyNi4xOTA0NTYzOTAzODA4Niwid2lkdGgiOjY3LjI0OTE2MDc2NjYwMTU2LCJoZWlnaHQiOjEuNTUxODM4MDQwMzUxODY3N319LHsicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjo4OS41NDQ0MDMwNzYxNzE4OCwibGVmdCI6MjYuMTkwNDU2MzkwMzgwODYsIndpZHRoIjo2Ny4yNTA1NjQ1NzUxOTUzMSwiaGVpZ2h0IjoxLjU1MTgzODA0MDM1MTg2Nzd9fSx7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6OTAuODIyNTg2MDU5NTcwMzEsImxlZnQiOjI2LjE5MDQ1NjM5MDM4MDg2LCJ3aWR0aCI6NjcuMjQxMTk1Njc4NzEwOTQsImhlaWdodCI6MS41NTE4MzgwNDAzNTE4Njc3fX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjkyLjEwMDc2OTA0Mjk2ODc1LCJsZWZ0IjoyNi4xOTA0NTYzOTAzODA4Niwid2lkdGgiOjY3LjI0Mjg0MzYyNzkyOTY5LCJoZWlnaHQiOjEuNTUxODM4MDQwMzUxODY3N319LHsicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjo0MS44MjAyNjY3MjM2MzI4MSwibGVmdCI6MjYuMTkwNDU2MzkwMzgwODYsIndpZHRoIjoxMS43ODM1NzQxMDQzMDkwODIsImhlaWdodCI6MS41OTIxMDA5Nzc4OTc2NDR9fSx7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6NzguMTgyNzc3NDA0Nzg1MTYsImxlZnQiOjI2LjE5MDQ1NjM5MDM4MDg2LCJ3aWR0aCI6NjYuMDg0NTAzMTczODI4MTIsImhlaWdodCI6Mi44NzAyODE2OTYzMTk1OH19LHsicGFnZU51bWJlciI6MTEsImJib3giOnsidG9wIjo3NC4yODg5MDIyODI3MTQ4NCwibGVmdCI6MjYuMTkwNDU2MzkwMzgwODYsIndpZHRoIjo2Ny4yNDQ1MDY4MzU5Mzc1LCJoZWlnaHQiOjIuODMwMDE4NzU4NzczODAzN319LHsicGFnZU51bWJlciI6MTEsImJib3giOnsidG9wIjo3Ni43ODk2OTU3Mzk3NDYxLCJsZWZ0IjoyOC4yMDYzMjc0MzgzNTQ0OTIsIndpZHRoIjo2NS4yMzIwNTU2NjQwNjI1LCJoZWlnaHQiOjEuNjA3NDA3NDUwNjc1OTY0NH19LHsicGFnZU51bWJlciI6MTEsImJib3giOnsidG9wIjo3OC4xMjM0NDM2MDM1MTU2MiwibGVmdCI6MjYuMTkwNDU2MzkwMzgwODYsIndpZHRoIjo2Ny4yNTM0MDI3MDk5NjA5NCwiaGVpZ2h0IjoxLjU1MTgzODA0MDM1MTg2Nzd9fSx7InBhZ2VOdW1iZXIiOjExLCJiYm94Ijp7InRvcCI6NzkuNDAxNjI2NTg2OTE0MDYsImxlZnQiOjI2LjE5MDQ1NjM5MDM4MDg2LCJ3aWR0aCI6NjcuMjUwMjI4ODgxODM1OTQsImhlaWdodCI6MS41NTE4MzgwNDAzNTE4Njc3fX0seyJwYWdlTnVtYmVyIjoxMSwiYmJveCI6eyJ0b3AiOjgwLjY3OTgwMTk0MDkxNzk3LCJsZWZ0IjoyNi4xOTA0NTYzOTAzODA4Niwid2lkdGgiOjY3LjI0NTczNTE2ODQ1NzAzLCJoZWlnaHQiOjEuNTUxODM4MDQwMzUxODY3N319LHsicGFnZU51bWJlciI6MTEsImJib3giOnsidG9wIjo4MS45NTc5ODQ5MjQzMTY0LCJsZWZ0IjoyNi4xOTA0NTYzOTAzODA4Niwid2lkdGgiOjY3LjI1MTczOTUwMTk1MzEyLCJoZWlnaHQiOjEuNTUxODM4MDQwMzUxODY3N319LHsicGFnZU51bWJlciI6MTEsImJib3giOnsidG9wIjo4My4yMzYxNjAyNzgzMjAzMSwibGVmdCI6MjYuMTkwNDU2MzkwMzgwODYsIndpZHRoIjo2Ny4yNTY3MzY3NTUzNzExLCJoZWlnaHQiOjEuNTUxODM4MDQwMzUxODY3N319XX0sImNodW5rSWQiOiIyNWUyM2JmYy1jYjY3LTRiYjItOGExYS1kNjIzN2YzM2Y4MDIiLCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIxMSJ9fV0%3D. This electrostatic stabilization mechanism is further evidenced by the tendency for zeta potential negativity to increase with solution pH, resulting in greater bubble surface charge and enhanced stability due to repulsive forces (Wu et al., 2021)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjMwLCJzdWJ0eXBlIjp7fSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSmlhamlhIiwiZmFtaWx5IjoiV3UifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiS2VqaWEiLCJmYW1pbHkiOiJaaGFuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJDaGVuZyIsImZhbWlseSI6IkNlbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJYaWFvZ2FuZyIsImZhbWlseSI6Ild1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlJ1eWluIiwiZmFtaWx5IjoiTWFvIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Illpbmd5aW5nIiwiZmFtaWx5IjoiWmhlbmcifX1dLCJpZHMiOnsiaXNzbiI6WyIyMTkxLTA4NTUiXSwiaXNibiI6W10sImRvaSI6IjEwLjExODYvczEzNTY4LTAyMS0wMTI1NC0wIiwibWFnIjoiMzE3NTY4MzU2MSIsInBtaWQiOiIzNDE4NDEzNyIsInBtY2lkIjoiODIzOTEwOSIsIm9wZW5hbGV4IjoiVzMxNzU2ODM1NjEiLCJvcGVuQWNjZXNzSWQiOiI4NzkxMTY1NzkxMjc0ODY4MTY1In0sInRpdGxlIjoiUm9sZSBvZiBidWxrIG5hbm9idWJibGVzIGluIHJlbW92aW5nIG9yZ2FuaWMgcG9sbHV0YW50cyBpbiB3YXN0ZXdhdGVyIHRyZWF0bWVudCIsImFic3RyYWN0IjoiQWJzdHJhY3QgVGhlIG9jY3VycmVuY2Ugb2YgYSB2YXJpZXR5IG9mIG9yZ2FuaWMgcG9sbHV0YW50cyBoYXMgY29tcGxpY2F0ZWQgd2FzdGV3YXRlciB0cmVhdG1lbnQ7IHRodXMsIHRoZSBzZWFyY2ggZm9yIHN1c3RhaW5hYmxlIGFuZCBlZmZlY3RpdmUgdHJlYXRtZW50IHRlY2hub2xvZ3kgaGFzIGRyYXduIHNpZ25pZmljYW50IGF0dGVudGlvbi4gSW4gcmVjZW50IHllYXJzLCBidWxrIG5hbm9idWJibGVzLCB3aGljaCBoYXZlIGV4dHJhb3JkaW5hcnkgcHJvcGVydGllcyBkaWZmZXJpbmcgZnJvbSB0aG9zZSBvZiBtaWNyb2J1YmJsZXMsIGluY2x1ZGluZyBoaWdoIHN0YWJpbGl0eSBhbmQgbG9uZyByZXNpZGVuY2UgdGltZXMgaW4gd2F0ZXIsIGxhcmdlIHNwZWNpZmljIHN1cmZhY2UgYXJlYXMsIGhpZ2ggZ2FzIHRyYW5zZmVyIGVmZmljaWVuY3kgYW5kIGludGVyZmFjZSBwb3RlbnRpYWwsIGFuZCB0aGUgY2FwYWJpbGl0eSB0byBnZW5lcmF0ZSBmcmVlIHJhZGljYWxzLCBoYXZlIHNob3duIGF0dHJhY3RpdmUgdGVjaG5vbG9naWNhbCBhZHZhbnRhZ2VzIGFuZCBwcm9taXNpbmcgYXBwbGljYXRpb24gcHJvc3BlY3RzIGZvciB3YXN0ZXdhdGVyIHRyZWF0bWVudC4gSW4gdGhpcyByZXZpZXcsIHRoZSBiYXNpYyBjaGFyYWN0ZXJpc3RpY3Mgb2YgYnVsayBuYW5vYnViYmxlcyBhcmUgc3VtbWFyaXplZCBpbiBkZXRhaWwsIGFuZCByZWNlbnQgZmluZGluZ3MgcmVsYXRlZCB0byB0aGVpciBpbXBsZW1lbnRhdGlvbiBwYXRod2F5cyBhbmQgbWVjaGFuaXNtcyBpbiBvcmdhbmljIHdhc3Rld2F0ZXIgdHJlYXRtZW50IGFyZSBzeXN0ZW1hdGljYWxseSBkaXNjdXNzZWQsIHdoaWNoIGluY2x1ZGVzIGltcHJvdmluZyB0aGUgYWlyIGZsb3RhdGlvbiBwcm9jZXNzLCBpbmNyZWFzaW5nIHdhdGVyIGFlcmF0aW9uIHRvIHByb21vdGUgYWVyb2JpYyBiaW9sb2dpY2FsIHRlY2hub2xvZ2llcyBpbmNsdWRpbmcgYmlvbG9naWNhbCBhY3RpdmF0ZWQgY2FyYm9uLCBhY3RpdmF0ZWQgc2x1ZGdlLCBhbmQgbWVtYnJhbmUgYmlvcmVhY3RvcnMsIGFuZCBnZW5lcmF0aW5nIGFjdGl2ZSBmcmVlIHJhZGljYWxzIHRoYXQgb3hpZGlzZSBvcmdhbmljIGNvbXBvdW5kcy4gRmluYWxseSwgdGhlIGN1cnJlbnQgdGVjaG5vbG9naWNhbCBkaWZmaWN1bHRpZXMgb2YgYnVsayBuYW5vYnViYmxlcyBhcmUgYW5hbHlzZWQsIGFuZCBmdXR1cmUgZm9jdXMgYXJlYXMgZm9yIHJlc2VhcmNoIG9uIGJ1bGsgbmFub2J1YmJsZSB0ZWNobm9sb2d5IGFyZSBhbHNvIHByb3Bvc2VkLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMTg2L3MxMzU2OC0wMjEtMDEyNTQtMCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9hbWItZXhwcmVzcy5zcHJpbmdlcm9wZW4uY29tL2NvdW50ZXIvcGRmLzEwLjExODYvczEzNTY4LTAyMS0wMTI1NC0wIiwiaXNzdWVkRGF0ZSI6IjIwMjEtMDYtMjgiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTEwIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJBTUIgRXhwcmVzcyIsImlzc3VlIjoxLCJ2b2x1bWUiOjExLCJpbXBhY3RGYWN0b3IiOiI0LjI2In0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlNwcmluZ2VyIE5hdHVyZSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NzYsImNpdGVkQnlDb3VudCI6NzV9fSwicXVvdGUiOiJUaGUgbW9kZWwgc3VjY2Vzc2Z1bGx5IGVsdWNpZGF0ZXMgcmVwdWxzaXZlIGZvcmNlcyAgYmV0d2VlbiBPSOKIkiBvbiB0aGUgc3VyZmFjZSBhcmUgc3VmZmljaWVudCB0byB1bHRpLW1hdGVseSBiYWxhbmNlIHRoZSBzdXJmYWNlIHRlbnNpb24gZm9yY2VzIGF0IG5hbm9idWJibGUgIHNpemVzLCB0aHVzLCBtYWtpbmcgbmFub2J1YmJsZXMgZXhoaWJpdCBncmVhdCBzdGFiaWwtaXR5LiBBcyBzaG93biBpbiBwcmV2aW91cyBsaXRlcmF0dXJlcywgemV0YSBwb3RlbnRpYWwgdmFsLXVlcyBjaGFuZ2Ugd2l0aCB0aGUgcEggb2YgdGhlIG1lZGl1bSB2YXJ5LCBleGhpYml0aW5nICBhIHRlbmRlbmN5IHRoYXQgdGhlIG5lZ2F0aXZpdHkgb2YgemV0YSBwb3RlbnRpYWwgaW5jcmVhc2VzICB3aXRoIHNvbHV0aW9uIHBIIGluY3JlYXNlIHdoaWNoIHJlc3VsdHMgaW4gZ3JlYXRlciBidWItYmxlIHN1cmZhY2UgY2hhcmdlIGFuZCBtYWtlIGJ1YmJsZXMgbW9yZSBzdGFibGUgZHVlIHRvICBlbGVjdHJvc3RhdGljIHJlcHVsc2lvbiAoTWVlZ29kYSBldCBhbC4gMjAxOCkuIE5hbm9idWItYmxlcyBnZW5lcmFsbHkgc2hvdyBoaWdoIG5lZ2F0aXZlIHpldGEgcG90ZW50aWFsIG93aW5nIHRvICB0aGUgZXhjZXNzIG9mIGh5ZHJveGlkZSBpb25zIChPSOKIkikgcmVsYXRpdmUgdG8gaHlkcm9nZW4gIGlvbnMgKEgrKSBhdCB0aGUgZ2Fz4oCTd2F0ZXIgaW50ZXJmYWNlIG9mIGJ1YmJsZXMgdW5kZXIgIHdpZGUgcmFuZ2Ugb2Ygc29sdXRpb24gcEgsIGJ1dCBuYW5vYnViYmxlcyBmb3JtZWQgaW4gIGFjaWRpYyBlbnZpcm9ubWVudHMgYWx3YXlzIHNob3cgbG93ZXIgc3RhYmlsaXR5IGNvbS1wYXJlZCB3aXRoIGluIG5ldXRyYWwgb3IgYWxrYWxpbmUgY29uZGl0aW9ucyBiZWNhdXNlIGEgIGxhY2sgb2YgT0jiiJIgYXJlIHJlcXVpcmVkIHRvIHN0YWJpbGl6ZSB0aGUgZWxlY3RyaWMgZG91YmxlICBsYXllciAoTmlybWFsa2FyIGV0IGFsLiAyMDE4KSwgdGhlIHpldGEgcG90ZW50aWFsIG9mIG5hbm8tYnViYmxlcyBnZW5lcmF0ZWQgaW4gc3Ryb25nIGFjaWRpYyBtZWRpdW0gYmVsb3cgcEggMyAgdGVuZHMgdG8gYmUgcG9zaXRpdmUgYW5kIGxvdyB3aGljaCBjYXVzZXMgaGlnaGVyIHBvc3NpLWJpbGl0eSBmb3IgYnViYmxlIGNvYWxlc2NlbmNlIChaaGFuZyBldCBhbC4gMjAyMCkuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MTMuOTY4MzA1NTg3NzY4NTU1LCJsZWZ0Ijo1Mi41MzQzNDc1MzQxNzk2OSwid2lkdGgiOjM4LjMyNjkxNTc0MDk2NjgsImhlaWdodCI6MS44Mzc2NTY2MTcxNjQ2MTE4fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MTUuNDc3MzM0MDIyNTIxOTczLCJsZWZ0Ijo1MS4xOTA5NzEzNzQ1MTE3Miwid2lkdGgiOjM5LjI4NTE2Mzg3OTM5NDUzLCJoZWlnaHQiOjEuODQ2NTYxNTUxMDk0MDU1Mn19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjE3LjAwMzU2MjkyNzI0NjA5NCwibGVmdCI6NTEuMTkwNDM3MzE2ODk0NTMsIndpZHRoIjozOS42NjkxMDU1Mjk3ODUxNTYsImhlaWdodCI6MS44Mzc2NTY2MTcxNjQ2MTE4fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MTguNTIwMjc3MDIzMzE1NDMsImxlZnQiOjUxLjE5MDQzNzMxNjg5NDUzLCJ3aWR0aCI6MzkuMjg1Njk0MTIyMzE0NDUsImhlaWdodCI6MS44MzgyNjYyNTM0NzEzNzQ1fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MjAuMDM4MjExODIyNTA5NzY2LCJsZWZ0Ijo1MS4xOTA0MzczMTY4OTQ1Mywid2lkdGgiOjM5LjI4NTY5NDEyMjMxNDQ1LCJoZWlnaHQiOjEuODM3NjU2NjE3MTY0NjExOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjIxLjU1NTUzNjI3MDE0MTYsImxlZnQiOjUxLjE5MDQzNzMxNjg5NDUzLCJ3aWR0aCI6MzkuNjc1MjE2Njc0ODA0NjksImhlaWdodCI6MS44Mzc2NTY2MTcxNjQ2MTE4fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MjMuMDcyMjQ4NDU4ODYyMzA1LCJsZWZ0Ijo1MS4xOTA0MzczMTY4OTQ1Mywid2lkdGgiOjM5LjY3MDMyNjIzMjkxMDE1NiwiaGVpZ2h0IjoxLjgzNzY1NjYxNzE2NDYxMTh9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjoyNC41ODg5NjI1NTQ5MzE2NCwibGVmdCI6NTEuMTkwNDM3MzE2ODk0NTMsIndpZHRoIjozOS4yODU2OTQxMjIzMTQ0NSwiaGVpZ2h0IjoxLjgzODg3NTg4OTc3ODEzNzJ9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjoyNi4xMDc1MDc3MDU2ODg0NzcsImxlZnQiOjUxLjE5MDQzNzMxNjg5NDUzLCJ3aWR0aCI6MzkuNjcyMjUyNjU1MDI5MywiaGVpZ2h0IjoxLjgzNzY1NjYxNzE2NDYxMTh9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjoyNy42MjQyMjE4MDE3NTc4MTIsImxlZnQiOjUxLjE5MDQzNzMxNjg5NDUzLCJ3aWR0aCI6MzkuMjg1Njk0MTIyMzE0NDUsImhlaWdodCI6MS44MzgyNjYyNTM0NzEzNzQ1fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MjkuMTQyMTU2NjAwOTUyMTUsImxlZnQiOjUxLjE5MDQzNzMxNjg5NDUzLCJ3aWR0aCI6MzkuNjc1MjM1NzQ4MjkxMDE2LCJoZWlnaHQiOjEuODM3NjU2NjE3MTY0NjExOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjMwLjY1MDU3MzczMDQ2ODc1LCJsZWZ0Ijo1MS4xOTA0MzczMTY4OTQ1Mywid2lkdGgiOjM5LjY3MTcyNjIyNjgwNjY0LCJoZWlnaHQiOjEuODQ2NTU4ODA5MjgwMzk1NX19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjMyLjE2NzkwMDA4NTQ0OTIyLCJsZWZ0Ijo1MS4xODk4OTk0NDQ1ODAwOCwid2lkdGgiOjM5LjY2OTk0MDk0ODQ4NjMzLCJoZWlnaHQiOjEuODQ2NTU4ODA5MjgwMzk1NX19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjMzLjY5MzUxNTc3NzU4Nzg5LCJsZWZ0Ijo1MS4xOTA4NjA3NDgyOTEwMTYsIndpZHRoIjozOS42NzU3MjAyMTQ4NDM3NSwiaGVpZ2h0IjoxLjgzNzY1NDU5MDYwNjY4OTV9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjozNS4yMTAyMzU1OTU3MDMxMjUsImxlZnQiOjUxLjE5MDg2MDc0ODI5MTAxNiwid2lkdGgiOjM5LjI4NTI3MDY5MDkxNzk3LCJoZWlnaHQiOjEuODM4ODc0MTAxNjM4Nzk0fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MzYuNzI4Nzc4ODM5MTExMzMsImxlZnQiOjUxLjE5MDQzNzMxNjg5NDUzLCJ3aWR0aCI6MzkuNjY4OTcyMDE1MzgwODYsImhlaWdodCI6MS44Mzc2NTQ1OTA2MDY2ODk1fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MzguMjM3MTc4ODAyNDkwMjM0LCJsZWZ0Ijo1MS4xOTA0MzczMTY4OTQ1Mywid2lkdGgiOjM5LjY2ODgwNzk4MzM5ODQ0LCJoZWlnaHQiOjEuODQ2NTgwMDI4NTMzOTM1NX19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjM5Ljc2MjgyMTE5NzUwOTc2NiwibGVmdCI6NTEuMTg5OTUyODUwMzQxOCwid2lkdGgiOjM5LjI4NjE4MjQwMzU2NDQ1LCJoZWlnaHQiOjEuODM4MjYwNTMxNDI1NDc2fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6NDEuMjgwNzUwMjc0NjU4MiwibGVmdCI6NTEuMTkwNDM3MzE2ODk0NTMsIndpZHRoIjozOS42NzI3MzMzMDY4ODQ3NjYsImhlaWdodCI6MS44Mzc2NTQ1OTA2MDY2ODk1fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6NDIuNzk3NDcwMDkyNzczNDQsImxlZnQiOjUxLjE5MDQzNzMxNjg5NDUzLCJ3aWR0aCI6MzkuMjg1Njk0MTIyMzE0NDUsImhlaWdodCI6MS44MzgyNjA1MzE0MjU0NzZ9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo0NC4zMTUzOTkxNjk5MjE4NzUsImxlZnQiOjUxLjE5MDQzNzMxNjg5NDUzLCJ3aWR0aCI6MzkuMjg1Njk0MTIyMzE0NDUsImhlaWdodCI6MS44Mzc2NTQ1OTA2MDY2ODk1fX1dfSwiY2h1bmtJZCI6ImM2ZTFmZTNmLWVmOWItNDNiZC1hMjA3LTQ5ZjE4ZTg3ODQzNCIsImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjMifX1d.

Nanobubbles can be generated through various mechanisms, each tailored to specific scalability and energy requirements. Figure 1 provides a schematic overview of the major nanobubble generation routes discussed in this section. Hydrodynamic cavitation is one of the most common industrial methods, involving the forceful passage of water through constrictions or venturi nozzles to create low-pressure zones that induce bubble formation and collapse. This process generates shear forces that fragment larger bubbles into nanobubbles, often supersaturating the water with gases like oxygen or ozone. Ultrasonic cavitation employs high-frequency sound waves (typically 20-100 kHz) to produce alternating pressure cycles, leading to the formation, growth, and implosive collapse of bubbles. The extreme conditions during collapse-temperatures exceeding 5000 K and pressures up to 1000 atm-facilitate the nucleation of stable nanobubbles laden with ROS. Electrochemical production, meanwhile, relies on electrolysis of water or gas-sparged electrolytes, where anodic oxidation and cathodic reduction generate hydrogen or oxygen nanobubbles directly at electrode surfaces, offering precise control over bubble composition and size distribution. Furthermore, nanopore membrane techniques utilize porous substrates to force gas through liquid phases, creating nanobubbles via shear and pressure differentials (Montazeri et al., 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNleWVkIE1vaGFtbWFkIiwiZmFtaWx5IjoiTW9udGF6ZXJpIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5pY29sYXMiLCJmYW1pbHkiOiJLYWxvZ2VyYWtpcyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiLOk861z47Pgc6zzrnOv8%2BCIiwiZmFtaWx5IjoizprOv867zrvOuc%2BMz4DOv8%2BFzrvOv8%2BCIn19XSwiaWRzIjp7Imlzc24iOlsiMjA0NS0yMzIyIl0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDM4L3M0MTU5OC0wMjMtNDM4MDMtNiIsInBtaWQiOiIzNzc5NDEyNyIsIm9wZW5hbGV4IjoiVzQzODczMzI0MDciLCJvcGVuQWNjZXNzSWQiOiIyODM3MTg4MzMzNjAzNjQ5NjY4In0sInRpdGxlIjoiRWZmZWN0IG9mIGNoZW1pY2FsIHNwZWNpZXMgYW5kIHRlbXBlcmF0dXJlIG9uIHRoZSBzdGFiaWxpdHkgb2YgYWlyIG5hbm9idWJibGVzIiwiYWJzdHJhY3QiOiJUaGUgY29sbG9pZGFsIHN0YWJpbGl0eSBvZiBhaXIgbmFub2J1YmJsZXMgKE5Ccykgd2FzIHN0dWRpZWQgYXQgZGlmZmVyZW50IHRlbXBlcmF0dXJlcyAoMC0zMCDCsEMpIGFuZCBpbiB0aGUgcHJlc2VuY2Ugb2Ygc3VsZmF0ZXMsIHR5cGljYWxseSBmb3VuZCBpbiBtaW5pbmcgZWZmbHVlbnRzLCBpbiBhIHdpZGUgcmFuZ2Ugb2YgTmEyU080IGNvbmNlbnRyYXRpb25zICgwLjAwMSB0byAxIE0pLCBhbG9uZyB3aXRoIHRoZSBlZmZlY3Qgb2Ygc3VyZmFjdGFudHMgKHNvZGl1bSBkb2RlY3lsIHN1bGZhdGUpLCBjaGxvcmlkZSBzYWx0cyAoTmFDbCksIGFuZCBhY2lkL2Jhc2UgcmVhZ2VudHMgYXQgYSBwSCByYW5nZSBmcm9tIDQgdG8gOS4gVXNpbmcgYSBuYW5vYnViYmxlIGdlbmVyYXRvciBiYXNlZCBvbiBoeWRyb2R5bmFtaWMgY2F2aXRhdGlvbiwgMS4yIMOXIDEwOCBidWJibGVzL21MIHdpdGggYSB0eXBpY2FsIHJhZGl1cyBvZiA4NC42NiDCsSA3Ljg4IG5tIHdlcmUgZ2VuZXJhdGVkIGluIGRlaW9uaXplZCB3YXRlci4gTXVsdGlwbGUgZXZpZGVuY2UgaXMgcHJvdmlkZWQgdG8gcHJvdmUgdGhlaXIgcHJlc2VuY2UgaW4gc3VzcGVuc2lvbiwgaW5jbHVkaW5nIHRoZSBUeW5kYWxsIGVmZmVjdCwgZHluYW1pYyBsaWdodCBzY2F0dGVyaW5nLCBhbmQgbmFub3BhcnRpY2xlIHNpemUgYW5hbHlzaXMuIFpldGEgcG90ZW50aWFsIG1lYXN1cmVtZW50cyByZXZlYWxlZCB0aGF0IE5CcyBhcmUgbmVnYXRpdmVseSBjaGFyZ2VkIGV2ZW4gYWZ0ZXIgdHdvIG1vbnRocyAoZnJvbSAtIDE5LjQ4IMKxIDEuODkgdG8gLSAxMC4xMyDCsSAxLjcxIG1WKSwgc3VnZ2VzdGluZyB0aGF0IHRoZWlyIHN0YWJpbGl0eSBpcyBkdWUgdG8gdGhlIG5lZ2F0aXZlIGNoYXJnZSBvbiB0aGVpciBzdXJmYWNlLiBOQnMgd2VyZSBmb3VuZCB0byBiZSBtb3JlIHN0YWJsZSBpbiBhbGthbGluZSBzb2x1dGlvbnMgY29tcGFyZWQgdG8gYWNpZGljIG9uZXMuIEZ1cnRoZXIsIGxvdyBhbW91bnRzIG9mIGJvdGggY2hsb3JpZGUgYW5kIHN1bGZhdGUgZGlzc29sdmVkIHNhbHRzIGxlZCB0byBhIHJlZHVjdGlvbiBvZiB0aGUgc2l6ZSBvZiBOQnMuIEhvd2V2ZXIsIHdoZW4gaGlnaCBhbW91bnRzIG9mIGRpc3NvbHZlZCBzYWx0cyBhcmUgcHJlc2VudCwgTkJzIGFyZSBtb3JlIGxpa2VseSB0byBjb2FsZXNjZSwgYW5kIHRoZWlyIHNpemUgdG8gYmUgaW5jcmVhc2VkLiBGaW5hbGx5LCB0aGUgaW52ZXN0aWdhdGlvbiBvZiB0aGUgc3RhYmlsaXR5IG9mIGFpciBOQnMgYXQgbG93IHRlbXBlcmF0dXJlcyByZXZlYWxlZCBhIG5vbi1tb25vdG9uaWMgcmVsYXRpb25zaGlwIGJldHdlZW4gdGVtcGVyYXR1cmUgYW5kIE5CcyB1cG9uIGNvbnNpZGVyaW5nIHdhdGVyIHNlbGYtaW9uaXphdGlvbiBhbmQgaW9uIG1vYmlsaXR5LiBUaGlzIHJlc2VhcmNoIGFpbXMgdG8gb3BlbiBhIG5ldyBmcm9udGllciB0b3dhcmRzIHRoZSBhcHBsaWNhdGlvbiBvZiB0aGUgaGlnaGx5IGlubm92YXRpdmUgTkJzIHRlY2hub2xvZ3kgb24gdGhlIHRyZWF0bWVudCBvZiBtaW5pbmcsIG1pbmVyYWwsIGFuZCBtZXRhbCBwcm9jZXNzaW5nIGVmZmx1ZW50cywgd2hpY2ggYXJlIGNoYWxsZW5naW5nIGFxdWVvdXMgc29sdXRpb25zIGNvbnRhaW5pbmcgY2hsb3JpZGUgYW5kIHN1bGZhdGUgc3BlY2llcy4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAzOC9zNDE1OTgtMDIzLTQzODAzLTYiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vd3d3Lm5hdHVyZS5jb20vYXJ0aWNsZXMvczQxNTk4LTAyMy00MzgwMy02LnBkZiIsImlzc3VlZERhdGUiOiIyMDIzLTEwLTA0IiwiYWNjZXNzZWREYXRlIjoiMjAyNS0wOCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiU2NpZW50aWZpYyBSZXBvcnRzIiwiaXNzdWUiOjEsInZvbHVtZSI6MTMsImltcGFjdEZhY3RvciI6IjQuOTEifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiTmF0dXJlIFBvcnRmb2xpbyJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NjksImNpdGVkQnlDb3VudCI6MjZ9fSwicXVvdGUiOiJUaGV5IGNhbiBiZSBjYXRlZ29yaXplZCBhcyBzdXJmYWNlIE5CcywgaS5lLiwgZ2VuZXJhdGVkIGF0IHNvbGlk4oCTbGlxdWlkICBpbnRlcmZhY2VzLCBvciBidWxrIE5CcywgaS5lLiwgZm9ybWVkIGluIGJ1bGsgbGlxdWlkczQsNS4gTmFub2J1YmJsZXMgaGF2ZSBtYW55IGFwcGxpY2F0aW9uczYgZnJvbSB3YXN0ZXdhdGVyICB0cmVhdG1lbnQsIGVjb3N5c3RlbSByZXN0b3JhdGlvbiwgc3VyZmFjZSBjbGVhbmluZywgb3h5Z2VuYXRpb24gaW4gYWdyaWN1bHR1cmUgYW5kIGluIGFxdWFjdWx0dXJlIHRvIGRpc2luZmVjLXRpb24sIGFuZCBtZWRpY2luZSwgaW5jbHVkaW5nIGluZHVzdHJpYWwgYXBwbGljYXRpb25zNyB0aGF0IGNvdWxkIGJlIHVzZWQgaW4gdGhlIGN1cnJlbnQgZGVjYXJib25pemF0aW9uIGVmZm9ydHM4ICBhbmQgdGhlIHRyYW5zaXRpb24gb2Ygb3VyIG1vZGVybiBzb2NpZXRpZXMgdG93YXJkcyBsb3ctY2FyYm9uIGVsZWN0cmljIGVjb25vbWllcy4gTmFub2J1YmJsZXMgYXJlIGNvbW1vbmx5ICBnZW5lcmF0ZWQgYnkgY2F2aXRhdGlvbiwgd2hpY2ggb2NjdXJzIHdoZW4gcHJlc3N1cmUgZmFsbHMgYmVsb3cgYSBwYXJ0aWN1bGFyIHRocmVzaG9sZCB2YWx1ZSwgdHlwaWNhbGx5IHZpYSBhY291cy10aWMsIG9wdGljYWwsIHBhcnRpY2xlLCBvciBoeWRyb2R5bmFtaWMgcHJlc3N1cmUgcmVkdWN0aW9uIHRlY2huaXF1ZXM54oCTMTYuIEZ1cnRoZXJtb3JlLCBzb25vY2hlbWlzdHJ5IHVzaW5nICAgdWx0cmFzb3VuZDE3LCBlbGVjdHJvbHlzaXMxOCwgYW5kIGFwcGx5aW5nIG5hbm9wb3JlIG1lbWJyYW5lczE5IGhhdmUgYmVlbiB1c2VkIHRvIGdlbmVyYXRlIE5Ccy4gRnVuZGFtZW50YWxseSwgIHRoZSBpbnRlcm5hbCBwcmVzc3VyZSBvZiBhIGJ1YmJsZSBncm93cyBzdWJzdGFudGlhbGx5IGFzIGl0cyBkaWFtZXRlciBkZWNyZWFzZXMsIGFjY29yZGluZyB0byB0aGUgWW91bmfigJNMYXBsYWNlICBlcXVhdGlvbiBbRXEuICgxKV0yMCwyMTogd2hlcmUgUGksIFBhLCDPgywgYW5kIGQgcmVwcmVzZW50IHRoZSBpbnRlcm5hbCBwcmVzc3VyZSBvZiB0aGUgYnViYmxlLCB0aGUgYXRtb3NwaGVyaWMgcHJlc3N1cmUsIHRoZSBzdXJmYWNlIHRlbi1zaW9uIG9mIHRoZSBsaXF1aWQsIGFuZCB0aGUgYnViYmxlIGRpYW1ldGVyLCByZXNwZWN0aXZlbHkuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NTguODQ0OTAyMDM4NTc0MjIsImxlZnQiOjI2LjE5MzM0OTgzODI1NjgzNiwid2lkdGgiOjY3LjI1MDU2NDU3NTE5NTMxLCJoZWlnaHQiOjEuNTU2MjY1MzU0MTU2NDk0MX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjYwLjEyMzA4NTAyMTk3MjY1NiwibGVmdCI6MjYuMTkwMDM2NzczNjgxNjQsIndpZHRoIjo2Ny4yNDg1NTA0MTUwMzkwNiwiaGVpZ2h0IjoxLjU1NjI2NTM1NDE1NjQ5NDF9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2MS40MDU1NjMzNTQ0OTIxOSwibGVmdCI6MjYuMTkxMDc0MzcxMzM3ODksIndpZHRoIjo2Ni45MDQ3NTQ2Mzg2NzE4OCwiaGVpZ2h0IjoxLjU1MTgzODA0MDM1MTg2Nzd9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2Mi42Nzk0NDcxNzQwNzIyNjYsImxlZnQiOjI2LjE5MTA2Njc0MTk0MzM2LCJ3aWR0aCI6NjcuMjQ3MzIyMDgyNTE5NTMsImhlaWdodCI6MS41NTYyNjUzNTQxNTY0OTQxfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NjMuOTYxOTIxNjkxODk0NTMsImxlZnQiOjI2LjE5MDQxODI0MzQwODIwMywid2lkdGgiOjY3LjI0MjA0MjU0MTUwMzksImhlaWdodCI6MS41NTE4NDE5NzQyNTg0MjI5fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NjUuMjM5OTc0OTc1NTg1OTQsImxlZnQiOjI2LjE5MDQxODI0MzQwODIwMywid2lkdGgiOjY2Ljg5NDAzNTMzOTM1NTQ3LCJoZWlnaHQiOjEuNTUxODM4MDQwMzUxODY3N319LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjY2LjUxMzk4NDY4MDE3NTc4LCJsZWZ0IjoyNi4xOTA0MTgyNDM0MDgyMDMsIndpZHRoIjo2Ny4yNTAyMzY1MTEyMzA0NywiaGVpZ2h0IjoxLjU1NjI2OTI4ODA2MzA0OTN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2Ny43OTIxNjc2NjM1NzQyMiwibGVmdCI6MjYuMTkwNzQwNTg1MzI3MTUsIndpZHRoIjo2Ny4yNDc1OTY3NDA3MjI2NiwiaGVpZ2h0IjoxLjU1NjI2OTI4ODA2MzA0OTN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2OS4wNzQ2NDU5OTYwOTM3NSwibGVmdCI6MjYuMTkwOTQ0NjcxNjMwODYsIndpZHRoIjo2Ny4yNDQ4NDI1MjkyOTY4OCwiaGVpZ2h0IjoxLjU1MTgzODA0MDM1MTg2Nzd9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo3MC4zNDg1MjYwMDA5NzY1NiwibGVmdCI6MjYuMTkwOTQ0NjcxNjMwODYsIndpZHRoIjoxMy4zNDMyNTQwODkzNTU0NjksImhlaWdodCI6MS41NTYyNjkyODgwNjMwNDkzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NzUuOTEyNjU4NjkxNDA2MjUsImxlZnQiOjI2LjE5MDEzNDA0ODQ2MTkxNCwid2lkdGgiOjY2LjkxODA0NTA0Mzk0NTMxLCJoZWlnaHQiOjEuNjM3MzQ5MTI4NzIzMTQ0NX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjc3LjE5MTA5MzQ0NDgyNDIyLCJsZWZ0IjoyNi4xOTA5OTYxNzAwNDM5NDUsIndpZHRoIjo2Ny4yNDg2MzQzMzgzNzg5LCJoZWlnaHQiOjEuNTUxODM4MDQwMzUxODY3N319XX0sImNodW5rSWQiOiI1MDIzMzFhYi1kNWZkLTQ0NTItYjY1ZS1mNTE2MDViMTdmMGUiLCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIxIn19XQ%3D%3D. These generation strategies result in bulk nanobubbles that fill the liquid volume, as opposed to surface nanobubbles which exist only at solid-liquid interfaces (Svetovoy, 2021)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlYuIEIuIiwiZmFtaWx5IjoiU3ZldG92b3kifX1dLCJpZHMiOnsiaXNzbiI6WyIxMzU5LTAyOTQiLCIxODc5LTAzOTkiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMTYvai5jb2Npcy4yMDIxLjEwMTQyMyIsIm1hZyI6IjMxMjY0ODQ4NDgiLCJvcGVuYWxleCI6IlczMTI2NDg0ODQ4Iiwib3BlbkFjY2Vzc0lkIjoiMTQ2MDczOTA4NTUzMzM1NDY5MzQifSwidGl0bGUiOiJTcG9udGFuZW91cyBjaGVtaWNhbCByZWFjdGlvbnMgYmV0d2VlbiBoeWRyb2dlbiBhbmQgb3h5Z2VuIGluIG5hbm9idWJibGVzIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjEwMTYvai5jb2Npcy4yMDIxLjEwMTQyMyIsInJlc291cmNlVXJsIjoiaHR0cDovL2FyeGl2Lm9yZy9wZGYvMjEwMi4wMzEyNiIsImlzc3VlZERhdGUiOiIyMDIxLTAyLTA1IiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiQ3VycmVudCBPcGluaW9uIGluIENvbGxvaWQgJiBJbnRlcmZhY2UgU2NpZW5jZSIsInZvbHVtZSI6NTJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjEwMTQyMywibGFzdFBhZ2UiOjEwMTQyM319LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJFbHNldmllciBCViJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NjEsImNpdGVkQnlDb3VudCI6Mn19LCJxdW90ZSI6IlRoaXMgYXR0ZW50aW9uIGlzIGRyaXZlbiBieSBhIGZ1bmRhLW1lbnRhbCBwcm9ibGVtOiBOQnMgbGl2ZSBtdWNoIHRvbyBsb25nIGluIGNvbXBhcmktc29uIHdpdGggdGhlIHByZWRpY3Rpb24gZ2l2ZW4gYnkgdGhlIGRpZmZ1c2lvbiBkaXNzb2x1LXRpb24gdGhlb3J5IFs0LCA1XS4gT24gdGhlIG90aGVyIGhhbmQsIGV4Y2l0aW5nIGFwcGxpY2EtdGlvbnMgb2YgTkJzIGluIGVudmlyb25tZW50IFs2LCA3XSwgYWdyaWN1bHR1cmUgWzhdLCBmbG90YS10aW9uIFs5LCAxMF0sIGhlYWx0aGNhcmUgWzExLCAxMl0sIGFuZCBjaGVtaXN0cnkgWzEzXSBoYXZlIGVtZXJnZWQgdG8gbmFtZSBvbmx5IGEgZmV3LiBJbiBjb250cmFzdCB3aXRoIHN1cmZhY2UgTkJzLCB3aGljaCBleGlzdCBvbiB0aGUgc29saWQtbGlxdWlkIGludGVyZmFjZSwgYnVsayBOQnMgZmlsbCB0aGUgdm9sdW1lIG9mIGxpcXVpZC4gVGhlc2UgTkJzIGFyZSBnZW5lcmF0ZWQgYnkgbW9zdGx5IG1lY2hhbmljYWwgaW5qZWN0aW9uIG9mIGRpZi1mZXJlbnQgZ2FzZXMgWzldIG9yIGJ5IHVsdHJhc29uaWMgY2F2aXRhdGlvbiBbMTBdLCBidXQgaHktZHJvZ2VuIGFuZCBveHlnZW4gTkJzIHdoZXJlIGFsc28gZ2VuZXJhdGVkIGJ5IERDIGVsZWN0cm9seXNpcyBbMTQsIDE1XS4gTGF0ZXIgdGhlc2UgbWV0aG9kcyB3aWxsIGJlIGNhbGxlZCB0aGUgcmVndWxhciBtZXRob2RzLiBUaGUgc2l6ZSBkaXN0cmlidXRpb24gb2YgdGhlIE5CcyB3YXMgY2hhcmFjdGVyaXplZCBieSB0aGUgZHluYW1pYyBsaWdodCBzY2F0dGVyaW5nIGFuZCBpbmRpdmlkdWFsIGJ1YmJsZXMgd2VyZSBvYnNlcnZlZCBieSBlbGVjdHJvbiBtaWNyb3Njb3B5IGZyb20gZnJlZXplIGZyYWN0dXJlIHJlcGxpY2FzIFsxNl0sIGJ5IHBoYXNlIG1pY3Jvc2NvcHkgYW5kIHBvbGFyaW1ldHJpYyBzY2F0dGVyb21ldHJ5IFsxN10uIFVuZXhwZWN0ZWRseSwgTkJzIGNvbnRhaW5pbmcgb3h5Z2VuIGFyZSBhYmxlIHRvIHByby1kdWNlIHJlYWN0aXZlIG94eWdlbiBzcGVjaWVzIHdoaWNoIGNhbiBveGlkaXplIHBvbGx1LXRhbnRzIGFuZCBwYXRob2dlbnMgaW4gd2F0ZXIgWzE44oCTMjBdLCB3aGlsZSBzZXBhcmF0ZSBveHktZ2VuIG1vbGVjdWxlcyBoYXZlIG5vIGVmZmVjdCBvbiBvcmdhbmljcy4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjozNi45NTA2NDU0NDY3NzczNDQsImxlZnQiOjguODIzNTI1NDI4NzcxOTczLCJ3aWR0aCI6NDAuMDUyMjYxMzUyNTM5MDYsImhlaWdodCI6MS4yNTc5MDU5NjAwODMwMDc4fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6MzguMzk3MjM1ODcwMzYxMzMsImxlZnQiOjguODIzNTI1NDI4NzcxOTczLCJ3aWR0aCI6NDAuMDQ4OTk5Nzg2Mzc2OTUsImhlaWdodCI6MS4yNTc5MDU5NjAwODMwMDc4fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6MzkuODQzODI2MjkzOTQ1MzEsImxlZnQiOjguODIzNTI1NDI4NzcxOTczLCJ3aWR0aCI6NDAuMDUyMjYxMzUyNTM5MDYsImhlaWdodCI6MS4yNTc5MDU5NjAwODMwMDc4fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NDEuMjkwNDE2NzE3NTI5MywibGVmdCI6OC44MjM1MjU0Mjg3NzE5NzMsIndpZHRoIjo0MC4wNTM4ODI1OTg4NzY5NSwiaGVpZ2h0IjoxLjI1NzkwNTk2MDA4MzAwNzh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo0Mi43MzcwMTA5NTU4MTA1NSwibGVmdCI6OC44MjM1MjU0Mjg3NzE5NzMsIndpZHRoIjo0MC4wNTIyNDYwOTM3NSwiaGVpZ2h0IjoxLjI1NzkwNTk2MDA4MzAwNzh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo0NC4xODM2MDEzNzkzOTQ1MywibGVmdCI6OC44MjM1MjU0Mjg3NzE5NzMsIndpZHRoIjo0MC4wNTIyNDYwOTM3NSwiaGVpZ2h0IjoxLjI1NzkwNTk2MDA4MzAwNzh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo0NS42MzAxODc5ODgyODEyNSwibGVmdCI6OC44MjM1MjU0Mjg3NzE5NzMsIndpZHRoIjoyMC41MzIzOTgyMjM4NzY5NTMsImhlaWdodCI6MS4yNTc5MDU5NjAwODMwMDc4fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NDcuMDc2NzgyMjI2NTYyNSwibGVmdCI6MTAuNDUxNDY2NTYwMzYzNzcsIndpZHRoIjozOC40MTk0OTg0NDM2MDM1MTYsImhlaWdodCI6MS4yNTc5MDU5NjAwODMwMDc4fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NDguNTIzMzcyNjUwMTQ2NDg0LCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA1Mzg4NjQxMzU3NDIyLCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjQ5Ljk2OTk2Njg4ODQyNzczNCwibGVmdCI6OC44MjM1MjU0Mjg3NzE5NzMsIndpZHRoIjo0MC4wNDczNzQ3MjUzNDE4LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjUxLjQxNjU1NzMxMjAxMTcyLCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA1MjI0NjA5Mzc1LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjUyLjg2MzE0MzkyMDg5ODQ0LCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA0NzM3NDcyNTM0MTgsImhlaWdodCI6MS4yNTc5MDU5NjAwODMwMDc4fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NTQuMzA5NzM4MTU5MTc5NjksImxlZnQiOjguODIzNTI1NDI4NzcxOTczLCJ3aWR0aCI6NDAuMDQ0MTEzMTU5MTc5NjksImhlaWdodCI6MS4yNTc5MDU5NjAwODMwMDc4fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NTUuNzU2MzMyMzk3NDYwOTQsImxlZnQiOjguODIzNTI1NDI4NzcxOTczLCJ3aWR0aCI6NDAuMDQ5MDExMjMwNDY4NzUsImhlaWdodCI6MS4yNTc5MDU5NjAwODMwMDc4fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NTcuMjAyOTE5MDA2MzQ3NjU2LCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA0NzM3NDcyNTM0MTgsImhlaWdodCI6MS4yNTc5MDU5NjAwODMwMDc4fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NTguNjQ5NTA5NDI5OTMxNjQsImxlZnQiOjguODIzNTI1NDI4NzcxOTczLCJ3aWR0aCI6NDAuMDQ5MDAzNjAxMDc0MjIsImhlaWdodCI6MS4yNTc5MDU5NjAwODMwMDc4fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NjAuMDk2MTAzNjY4MjEyODksImxlZnQiOjguODIzNTI1NDI4NzcxOTczLCJ3aWR0aCI6NDAuMDQ1NzQ1ODQ5NjA5Mzc1LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjYxLjU0MjY5MDI3NzA5OTYxLCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjI1LjcxMjMwMzE2MTYyMTA5NCwiaGVpZ2h0IjoxLjI1NzkwNTk2MDA4MzAwNzh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2Mi45ODk0MDY1ODU2OTMzNiwibGVmdCI6MTAuNDUxNDY2NTYwMzYzNzcsIndpZHRoIjozOC40MjExMTIwNjA1NDY4NzUsImhlaWdodCI6MS4yNTc5MDU5NjAwODMwMDc4fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NjQuNDM1OTk3MDA5Mjc3MzQsImxlZnQiOjguODIzNTI1NDI4NzcxOTczLCJ3aWR0aCI6NDAuMDQ1NzQ1ODQ5NjA5Mzc1LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjY1Ljg4MjU5MTI0NzU1ODYsImxlZnQiOjguODIzNTI1NDI4NzcxOTczLCJ3aWR0aCI6NDAuMDQ4OTk1OTcxNjc5NjksImhlaWdodCI6MS4yNTc5MDIxNDUzODU3NDIyfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NjcuMzI5MTkzMTE1MjM0MzgsImxlZnQiOjguODIzNTI1NDI4NzcxOTczLCJ3aWR0aCI6MzAuNjY5MTc5OTE2MzgxODM2LCJoZWlnaHQiOjEuMjU3OTAyMTQ1Mzg1NzQyMn19XX0sImNodW5rSWQiOiIxNDY4ODgwOC02ZDhkLTQyZDItYTU1NS1kMDNiYzJmZTU4N2UiLCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIxMDE0MjMifX1d.

Figure 1: Schematic representation of nanobubble generation methods

Other advanced techniques include pressurized dissolution followed by depressurization, where gases are dissolved under high pressure and then released, forming nanobubbles upon sudden expansion. These methods vary in energy consumption and output; for instance, hydrodynamic systems are cost-effective for large-scale aquaculture, while ultrasonic methods excel in lab settings for targeted ROS production. In aquaculture contexts, oxygen or ozone nanobubbles have shown promise in elevating dissolved oxygen levels and reducing total ammonia nitrogen by up to 9% in shrimp ponds, as demonstrated in controlled trials (Nghĩa et al., 2022; Susanti et al., 2021)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5ndXnhu4VuIEjhu691IiwiZmFtaWx5IjoiTmdoxKlhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5ndXllbiBUaGkiLCJmYW1pbHkiOiJOZ3V5ZW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUGhhbiBUcuG7jW5nIiwiZmFtaWx5IjoiQsOsbmgifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTGUgVGhpIiwiZmFtaWx5IjoiTWF5In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRvbmcgVHJhbiIsImZhbWlseSI6Ikh1eSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQaOG6oW0gVGjDoWkiLCJmYW1pbHkiOiJHaWFuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTb3BoaWUiLCJmYW1pbHkiOiJTdOKAkEhpbGFpcmUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUGhhbiBUaGkiLCJmYW1pbHkiOiJWYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMjM0LTE3NDkiLCIyMjM0LTE3NTciXSwiaXNibiI6W10sImRvaSI6IjEwLjQ3ODUzL2Zhcy4yMDIyLmUzOSIsIm9wZW5hbGV4IjoiVzQyOTQ1OTc1NTQifSwidGl0bGUiOiJFZmZlY3Qgb2YgbmFub2J1YmJsZXMgKG94eWdlbiwgb3pvbmUpIG9uIHRoZSBQYWNpZmljIHdoaXRlIHNocmltcCAoUGVuYWV1cyB2YW5uYW1laSksIFZpYnJpbyBwYXJhaGFlbW9seXRpY3VzIGFuZCB3YXRlciBxdWFsaXR5IHVuZGVyIGxhYiBjb25kaXRpb25zIiwiYWJzdHJhY3QiOiJUaGlzIHN0dWR5IGFzc2Vzc2VkIHRoZSBlZmZlY3RzIG9mIG94eWdlbiBhbmQgb3pvbmUgbmFub2J1YmJsZXMgb24gZ2lsbCBtb3JwaG9sb2d5LCB3ZWlnaHQgZ2FpbiwgYW5kIG1vcnRhbGl0eSBvZiBQYWNpZmljIHdoaXRlIHNocmltcCAoUGVuYWV1cyB2YW5uYW1laSksIGFzIHdlbGwgYXMgdGhlIGxldmVsIG9mIFZpYnJpbyBwYXJhaGFlbW9seXRpY3VzIGFuZCB3YXRlciBxdWFsaXR5IG9mIHNocmltcCBjdWx0dXJlIHRhbmtzIHVuZGVyIGxhYiBjb25kaXRpb25zLiBUd28gZXhwZXJpbWVudHMgd2VyZSBjYXJyaWVkIG91dCB3aXRoIG94eWdlbiBtYWNyb2J1YmJsZSwgb3pvbmUgbWFjcm9idWJibGUsIG94eWdlbiBuYW5vYnViYmxlLCBvem9uZSBuYW5vYnViYmxlLCBhbmQgY29udHJvbCB0cmVhdG1lbnRzIChhaXItc3RvbmUgbWFjcm9idWJibGUpLiBFeHBlcmltZW50cyB3ZXJlIGRvbmUgaW4gdHJpcGxpY2F0ZSBpbiAxMDAgTCB0YW5rcyB3aXRoIDE14oCwIHNhbGluZSB3YXRlciwgYW5kIDIwIHNocmltcCBwZXIgdGFuay4gVGFua3MgaW4gRXhwZXJpbWVudCAxIHdlcmUgbm90IGlub2N1bGF0ZWQgd2l0aCBiYWN0ZXJpYTsgdGFua3MgaW4gRXhwZXJpbWVudCAyIHdlcmUgaW5vY3VsYXRlZCB3aXRoIFYuIHBhcmFoYWVtb2x5dGljdXMgYXQgYSBjb25jZW50cmF0aW9uIG9mIDEwNiBDRlUvbUwuIFRoZSByZXN1bHRzIHJldmVhbGVkIHRoYXQgc2hvcnQgdHJlYXRtZW50cyB3aXRoIG96b25lIG5hbm9idWJibGVzIGhhZCBtaW5pbWFsIGltcGFjdCBvbiBzaHJpbXAgZ2lsbHMsIG1vcnRhbGl0eSwgYW5kIGdyb3d0aCByYXRlcywgcmVkdWNlZCBWLiBwYXJhaGFlbW9seXRpY3VzIGNvbmNlbnRyYXRpb24gaW4gd2F0ZXIgY29tcGFyZWQgdG8gdGhlIG90aGVyIGdyb3VwcywgYW5kIGltcHJvdmUgd2F0ZXIgcXVhbGl0eS4gVGhlc2UgbGFib3JhdG9yeSByZXN1bHRzIGluZGljYXRlIHRoYXQgb3pvbmUgbmFub2J1YmJsZSB0cmVhdG1lbnQgbWF5IGJlIHVzZWZ1bCBmb3IgY29udHJvbGxpbmcgVi4gcGFyYWhhZW1vbHl0aWN1cy4gTW9yZSB3b3JrIGlzIG5lZWRlZCB0byBmaW5kIHRoZSBiZXN0IHByb3RvY29sIHRvIGFwcGx5IHRoZSB0ZWNobm9sb2d5IG9uIGEgY29tbWVyY2lhbCBzY2FsZS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuNDc4NTMvZmFzLjIwMjIuZTM5IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5lLWZhcy5vcmcvZG93bmxvYWQvZG93bmxvYWRfcGRmP3BpZD1mYXMtMjUtOC00MjkiLCJpc3N1ZWREYXRlIjoiMjAyMi0wOC0wMSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZpc2hlcmllcyBhbmQgYXF1YXRpYyBzY2llbmNlcyIsImlzc3VlIjo4LCJ2b2x1bWUiOjI1LCJpbXBhY3RGYWN0b3IiOiIxLjM0In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6NDI5LCJsYXN0UGFnZSI6NDQwfX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlNwcmluZ2VyIFNjaWVuY2UrQnVzaW5lc3MgTWVkaWEifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjMwLCJjaXRlZEJ5Q291bnQiOjEzfX0sInF1b3RlIjoiVGhlIGluZHVzdHJ5IHRoZXJlZm9yZSBuZWVkcyB0byBmaW5kIGFsdGVybmF0aXZlIG1lYXN1cmVzICB0byBwcmV2ZW50IGFuZCBjb250cm9sIHRoaXMgcGF0aG9nZW4uIE5hbm9idWJibGUgdGVjaG5vbG9neSBwcm9kdWNlcyBzbWFsbCBnYXMgYnViYmxlcyB3aXRoIGEgZGlhbWV0ZXIgb2YgbGVzcyB0aGFuIDIwMCBubSAoQWdyYXdhbCBldCBhbC4sIDIwMTE7IENoYXBsaW4sIDIwMjEpIHdoaWNoIGNhbiBleGlzdCBpbiB0aGUgd2F0ZXIgY29sdW1uIGZvciB3ZWVrcyAoQXpldmVkbyAgZXQgYWwuLCAyMDE2OyBQYXJtYXIgJiBNYWp1bWRlciwgMjAxMykuIFRoaXMgcGhlbm9tZW5vbiBpcyAgYXNzb2NpYXRlZCB3aXRoIHRoZSBwaHlzaWNhbCBjaGFyYWN0ZXJpc3RpYyBvZiB0aGVzZSBzbWFsbCBidWItYmxlcyAoV2VpanMgJiBMb2hzZSwgMjAxMykgYW5kIGhpZ2ggemV0YSBwb3RlbnRpYWwgdmFsdWUgd2hpY2ggIGtlZXBzIHRoZW0gZnJvbSBhZ2dyZWdhdGluZyBpbnRvIGxhcmdlciBidWJibGVzIChLaXJieSwgMjAxMDsgIFNqb2dyZWVuIGV0IGFsLiwgMjAxOCkuIE5hbm9idWJibGUgdGVjaG5vbG9neSBoYXMgYSBsb3Qgb2YgcHJvbS1pc2luZyBlZmZlY3RpdmUgYXBwbGljYXRpb24gaW4gYXF1YWN1bHR1cmUgYmVjYXVzZSBvZiBpdHMgYWJpbGl0eSAgdG8gaW5jcmVhc2UgYW5kIG1haW50YWluIG94eWdlbiBjb250ZW50IGluIHdhdGVyIGFuZCBjb250cm9sICBiYWN0ZXJpYSwgYSBwb3NzaWJsZSBtZWFzdXJlIHRvIHJlZHVjZSB0aGUgdXNlIG9mIGFudGliaW90aWNzLiBPeHlnZW4gbmFub2J1YmJsZXMgb3B0aW1pemUgdGhlIGRpc3NvbHZlZCBveHlnZW4gaW4gYXF1YXRpYyBzeXN0ZW1zIChHYWxhbmcgZXQgYWwuLCAyMDE5KS4gSGlnaCBsZXZlbHMgb2Ygb3h5Z2VuIG1heSBzcGVlZCB1cCBtZXRhYm9saXNtLCBwcm9tb3RlIGhpZ2hlciBmb29kIGludGFrZSwgYW5kIGluY3JlYXNlICBncm93dGggb2YgZmFybWVkIHNocmltcCAoRWJpbmEgZXQgYWwuLCAyMDEzKS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNS4wNjg2NDU0NzcyOTQ5MiwibGVmdCI6OC4wOTUyMzU4MjQ1ODQ5NjEsIndpZHRoIjo0MS4wNzIyMTIyMTkyMzgyOCwiaGVpZ2h0IjoxLjY5OTYzNTE0ODA0ODQwMDl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNi44MzI1MzA5NzUzNDE4LCJsZWZ0Ijo4LjA5NTIzNTgyNDU4NDk2MSwid2lkdGgiOjIzLjYwNDE4NTEwNDM3MDExNywiaGVpZ2h0IjoxLjY5OTYzNTE0ODA0ODQwMDl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC41OTY0MjAyODgwODU5NCwibGVmdCI6MTAuOTUyMzY0OTIxNTY5ODI0LCJ3aWR0aCI6MzguMjE1ODc3NTMyOTU4OTg0LCJoZWlnaHQiOjEuNjk5NjM1MTQ4MDQ4NDAwOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQwLjM2MDMwOTYwMDgzMDA4LCJsZWZ0Ijo4LjA5NTIzNTgyNDU4NDk2MSwid2lkdGgiOjQxLjA3Njc1OTMzODM3ODkwNiwiaGVpZ2h0IjoxNy41NzQ2MzA3MzczMDQ2ODh9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1Ny45OTkxOTUwOTg4NzY5NSwibGVmdCI6MTAuOTUyMzY0OTIxNTY5ODI0LCJ3aWR0aCI6MzguMjAxNDQ2NTMzMjAzMTI1LCJoZWlnaHQiOjEuNjk5NjM1MTQ4MDQ4NDAwOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjU5Ljc2MzA4MDU5NjkyMzgzLCJsZWZ0Ijo4LjA5NTIzNTgyNDU4NDk2MSwid2lkdGgiOjQxLjA2MzMyNzc4OTMwNjY0LCJoZWlnaHQiOjEuNjk5NjM1MTQ4MDQ4NDAwOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjYxLjUyNjk3MzcyNDM2NTIzNCwibGVmdCI6OC4wOTUyMzU4MjQ1ODQ5NjEsIndpZHRoIjo0MS4wNjY4NDQ5NDAxODU1NSwiaGVpZ2h0IjoxLjY5OTYzNTE0ODA0ODQwMDl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2My4yOTA4NjMwMzcxMDkzNzUsImxlZnQiOjguMDk1MjM1ODI0NTg0OTYxLCJ3aWR0aCI6NDAuNzE0MjkwNjE4ODk2NDg0LCJoZWlnaHQiOjEuNjk5NjMxMzMzMzUxMTM1M319XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjQzMCJ9fSx7Im9yaWdpbiI6NCwid29yayI6eyJ0eXBlIjo4LCJzdWJ0eXBlIjp7Im9uZW9mS2luZCI6ImFydGljbGVUeXBlIiwiYXJ0aWNsZVR5cGUiOjF9LCJjb250cmlidXRvcnMiOlt7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJMaWx5IiwiZmFtaWx5IjoiU3VzYW50aSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTdXl1ZCBXYXJubyIsImZhbWlseSI6IlV0b21vIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5vdmVyaXRhIERpYW4iLCJmYW1pbHkiOiJUYWthcmluYSJ9fV0sImlkcyI6eyJpc3NuIjpbIjIxMTctNDQ1OCIsIjIyNzMtMTcwOSJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMTA1MS9iaW9jb25mLzIwMjEzMzA1MDA1IiwibWFnIjoiMzE5MzU2MzcwMyIsIm9wZW5hbGV4IjoiVzMxOTM1NjM3MDMifSwidGl0bGUiOiJTdXN0YWluYWJpbGl0eSBhbmQgZmVhc2liaWxpdHkgYXNzZXNzbWVudHMgb2YgbmFub2J1YmJsZSBhZXJhdGlvbiB0ZWNobm9sb2d5IGluIGVjb25vbWljLXNvY2lvIGVudmlyb25tZW50IG9mIFBlbmFldXMgdmFubmFtZWkgc2hyaW1wIGZhcm1pbmciLCJhYnN0cmFjdCI6IlBlbmFldXMgdmFubmFtZWkgc2hyaW1wIGZhcm1pbmcgaW4gSW5kb25lc2lhIGlzIGZhY2VkIHdpdGggc2V2ZXJhbCBjaGFsbGVuZ2VzIGluIHRoZSBlbnZpcm9ubWVudGFsLCBlY29ub21pYywgYW5kIHNvY2lhbCBhc3BlY3RzLiBUaGVyZWZvcmUsIHRoaXMgc3R1ZHkgYWltcyB0byBhc3Nlc3MgdGhlIGJlbmVmaXRzIG9mIG5vdmVsIG5hbm9idWJibGUgYWVyYXRpb24gc3lzdGVtcyBmb3IgdGhlIGVjb3N5c3RlbSwgYnVzaW5lc3NlcywgYW5kIGNvbW11bml0aWVzLiBUaGlzIGlzIGFuIGV4cGVyaW1lbnRhbCBzdHVkeSBjb25kdWN0ZWQgYnkgY29tcGFyaW5nIFAuIHZhbm5hbWVpIHBvc3QgbGFydmEgMTAgcmVhcmVkIGF0IGEgZGVuc2l0eSBvZiAyMDAwIGluZHMuL0wgaW4gYSBwb25kIHRyZWF0ZWQgd2l0aCBuYW5vYnViYmxlIGFuZCB3aXRob3V0IHRyZWF0bWVudHMgKGNvbnRyb2wpLiBGdXJ0aGVybW9yZSwgdGhlIE5ldCBQcmVzZW50IFZhbHVlIChOUFYpIGFuZCBwYXliYWNrIHBlcmlvZCB3ZXJlIGNhbGN1bGF0ZWQgdG8gYXNzZXNzIHRoZSBlY29ub21pYyBmZWFzaWJpbGl0eSBvZiBuYW5vYnViYmxlLCB3aGlsZSBjb21tdW5pdHkgaW50ZXJ2aWV3cyB3ZXJlIHVzZWQgZm9yIHRoZSBzb2NpYWwgc2VjdG9ycy4gQmFzZWQgb24gdGhlIHJlc3VsdHMsIGVudmlyb25tZW50YWwgd2FzdGVzIG9mIHNocmltcCBmYXJtaW5nIGluIGZvcm0gb2YgdG90YWwgYW1tb25pYSBuaXRyb2dlbiAoVEFOKSB3ZXJlIHJlZHVjZWQgYnkgOSUgZnJvbSAyLjU4IG1nL2wgKDk1JUNJOiAwLDkxLCA0LDI1KSBpbiBjb250cm9sIGNvbXBhcmVkIHRvIDIuMzUgbWcvbCAoOTUlQ0k6IDAsODYsIDMsODQpIGluIHRyZWF0bWVudC4gRnVydGhlcm1vcmUsIHRoZSByZXZlbnVlcyBmcm9tIHBvc3QgbGFydmEgc2FsZXMgZm9yIGZpdmUgeWVhcnMgYW5kIG5hbm9idWJibGUgaW52ZXN0bWVudCBjb3N0cyBzaG93ZWQgdGhhdCB0aGUgZXN0aW1hdGVkIE5QViB3YXMgSURSIDY0LDgyNCwzNzQgd2l0aCBhIHBheWJhY2sgcGVyaW9kIG9mIDEuNyB5ZWFycy4gVGhlIGludGVydmlld3Mgb24gdHJhZGl0aW9uYWwgc2hyaW1wIGZhcm1pbmcgc2hvd2VkIHRoYXQgNjEuMSUgdG8gNzIuMiUgb2YgY29tbXVuaXR5IG1lbWJlcnMgYWdyZWVkIG9uIHRoZSB1c2Ugb2YgbmFub2J1YmJsZSBhZXJhdGlvbiB0byBzdXBwb3J0IGxpdmVsaWhvb2RzLiBUaGVyZWZvcmUsIHRoZSB1c2Ugb2YgbmFub2J1YmJsZXMgaXMgZmVhc2libGUgdG8gc3VwcG9ydCBzdXN0YWluYWJsZSBQLiB2YW5uYW1laSBmYXJtaW5nLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDUxL2Jpb2NvbmYvMjAyMTMzMDUwMDUiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vd3d3LmJpby1jb25mZXJlbmNlcy5vcmcvYXJ0aWNsZXMvYmlvY29uZi9wZGYvMjAyMS8wNS9iaW9jb25mX2ljYXZlc3MyMDIxXzA1MDA1LnBkZiIsImlzc3VlZERhdGUiOiIyMDIxLTAxLTAxIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0wOSIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiQklPIFdlYiBvZiBDb25mZXJlbmNlcyIsInZvbHVtZSI6MzMsImltcGFjdEZhY3RvciI6IjAuNjIifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjo1MDA1LCJsYXN0UGFnZSI6NTAwNX19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJFRFAgU2NpZW5jZXMifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjExLCJjaXRlZEJ5Q291bnQiOjR9fSwicXVvdGUiOiJBYnN0cmFjdC4gUGVuYWV1cyB2YW5uYW1laSBzaHJpbXAgZmFybWluZyBpbiBJbmRvbmVzaWEgaXMgZmFjZWQgd2l0aCBzZXZlcmFsIGNoYWxsZW5nZXMgaW4gdGhlIGVudmlyb25tZW50YWwsIGVjb25vbWljLCBhbmQgc29jaWFsIGFzcGVjdHMuICBUaGVyZWZvcmUsIHRoaXMgc3R1ZHkgYWltcyB0byBhc3Nlc3MgdGhlIGJlbmVmaXRzIG9mIG5vdmVsIG5hbm9idWJibGUgIGFlcmF0aW9uIHN5c3RlbXMgZm9yIHRoZSBlY29zeXN0ZW0sIGJ1c2luZXNzZXMsIGFuZCBjb21tdW5pdGllcy4gVGhpcyBpcyBhbiAgZXhwZXJpbWVudGFsIHN0dWR5IGNvbmR1Y3RlZCBieSBjb21wYXJpbmcgUC4gdmFubmFtZWkgcG9zdCBsYXJ2YSAxMCAgcmVhcmVkIGF0IGEgZGVuc2l0eSBvZiAyMDAwIGluZHMuL0wgaW4gYSBwb25kIHRyZWF0ZWQgd2l0aCBuYW5vYnViYmxlIGFuZCAgd2l0aG91dCB0cmVhdG1lbnRzIChjb250cm9sKS4gRnVydGhlcm1vcmUsIHRoZSBOZXQgUHJlc2VudCBWYWx1ZSAoTlBWKSBhbmQgIHBheWJhY2sgcGVyaW9kIHdlcmUgY2FsY3VsYXRlZCB0byBhc3Nlc3MgdGhlIGVjb25vbWljIGZlYXNpYmlsaXR5IG9mICBuYW5vYnViYmxlLCB3aGlsZSBjb21tdW5pdHkgaW50ZXJ2aWV3cyB3ZXJlIHVzZWQgZm9yIHRoZSBzb2NpYWwgc2VjdG9ycy4gIEJhc2VkIG9uIHRoZSByZXN1bHRzLCBlbnZpcm9ubWVudGFsIHdhc3RlcyBvZiBzaHJpbXAgZmFybWluZyBpbiBmb3JtIG9mICB0b3RhbCBhbW1vbmlhIG5pdHJvZ2VuIChUQU4pIHdlcmUgcmVkdWNlZCBieSA5JSBmcm9tIDIuNTggbWcvbCAoOTUlQ0k6ICAwLDkxLCA0LDI1KSBpbiBjb250cm9sIGNvbXBhcmVkIHRvIDIuMzUgbWcvbCAoOTUlQ0k6IDAsODYsIDMsODQpIGluICB0cmVhdG1lbnQuIEZ1cnRoZXJtb3JlLCB0aGUgcmV2ZW51ZXMgZnJvbSBwb3N0IGxhcnZhIHNhbGVzIGZvciBmaXZlIHllYXJzIGFuZCAgbmFub2J1YmJsZSBpbnZlc3RtZW50IGNvc3RzIHNob3dlZCB0aGF0IHRoZSBlc3RpbWF0ZWQgTlBWIHdhcyBJRFIgIDY0LDgyNCwzNzQgd2l0aCBhIHBheWJhY2sgcGVyaW9kIG9mIDEuNyB5ZWFycy4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo0NS40ODMyODc4MTEyNzkzLCJsZWZ0IjoyMS44Mjc3NzAyMzMxNTQyOTcsIndpZHRoIjo1Ni44MDA1Njc2MjY5NTMxMjUsImhlaWdodCI6MS43NDkyMzg2MTAyNjc2MzkyfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NDcuMTY3MTcxNDc4MjcxNDg0LCJsZWZ0IjoyMS44Mjc3NzAyMzMxNTQyOTcsIndpZHRoIjo1Ni43NzUwMzk2NzI4NTE1NiwiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo0OC42MjM4MDk4MTQ0NTMxMjUsImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2Ljc3MTMwMTI2OTUzMTI1LCJoZWlnaHQiOjEuNDA2Nzg1OTY0OTY1ODIwM319LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjUwLjA4MDQ0ODE1MDYzNDc2NiwibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuODIwMjk3MjQxMjEwOTQsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NTEuNTM3MDkwMzAxNTEzNjcsImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2LjgyNjUzMDQ1NjU0Mjk3LCJoZWlnaHQiOjEuNDA2Nzg1OTY0OTY1ODIwM319LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjUzLjAxMDY2NTg5MzU1NDY5LCJsZWZ0IjoyMS44Mjc3NzAyMzMxNTQyOTcsIndpZHRoIjo1Ni43NjU3MjQxODIxMjg5MDYsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NTQuNDY3MzA0MjI5NzM2MzMsImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2Ljc5MzcxMjYxNTk2NjgsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NTUuOTIzOTQ2MzgwNjE1MjM0LCJsZWZ0IjoyMS44Mjc3NzAyMzMxNTQyOTcsIndpZHRoIjo1Ni43OTU2MTYxNDk5MDIzNDQsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NTcuMzgwNTg4NTMxNDk0MTQsImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2Ljc3NjkyNzk0Nzk5ODA1LCJoZWlnaHQiOjEuNDA2Nzg1OTY0OTY1ODIwM319LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjU4Ljg1NDE2NDEyMzUzNTE1NiwibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuNzgwNjY2MzUxMzE4MzYsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NjAuMzEwNzk4NjQ1MDE5NTMsImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2LjgzMzM3NDAyMzQzNzUsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NjEuNzY3NDQwNzk1ODk4NDQsImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2Ljc0NzAwOTI3NzM0Mzc1LCJoZWlnaHQiOjEuNDA2Nzg1OTY0OTY1ODIwM319LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjYzLjIyODMxMzQ0NjA0NDkyLCJsZWZ0IjoyMS44Mjc3NzAyMzMxNTQyOTcsIndpZHRoIjo1Ni43ODgxMjc4OTkxNjk5MiwiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2NC43MDE4ODkwMzgwODU5NCwibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuNzg2MjYyNTEyMjA3MDMsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NjYuMTU4NTMxMTg4OTY0ODQsImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2LjgxNTMxOTA2MTI3OTMsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNTAwNSJ9fV0%3D.

The principles underlying nanobubble technology position it as a cornerstone for sustainable aquaculture, bridging fundamental physics with practical water management. By leveraging these generation methods, producers can achieve dynamic control over water chemistry without chemical additives, paving the way for reduced disease incidence and enhanced productivity. Specific implementations, such as the aQua+75MO generator, have demonstrated the capability to produce oxygen nanobubbles with an average size of 168.9 ± 73.8 nm and a concentration of 1.04 × 10⁹ ± 2.6 × 10⁸ particles/mL, utilizing a 1 HP motor to process water at a flow rate of 1,000 L/h (Nghĩa et al., 2022)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5ndXnhu4VuIEjhu691IiwiZmFtaWx5IjoiTmdoxKlhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5ndXllbiBUaGkiLCJmYW1pbHkiOiJOZ3V5ZW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUGhhbiBUcuG7jW5nIiwiZmFtaWx5IjoiQsOsbmgifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTGUgVGhpIiwiZmFtaWx5IjoiTWF5In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRvbmcgVHJhbiIsImZhbWlseSI6Ikh1eSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQaOG6oW0gVGjDoWkiLCJmYW1pbHkiOiJHaWFuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTb3BoaWUiLCJmYW1pbHkiOiJTdOKAkEhpbGFpcmUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUGhhbiBUaGkiLCJmYW1pbHkiOiJWYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMjM0LTE3NDkiLCIyMjM0LTE3NTciXSwiaXNibiI6W10sImRvaSI6IjEwLjQ3ODUzL2Zhcy4yMDIyLmUzOSIsIm9wZW5hbGV4IjoiVzQyOTQ1OTc1NTQiLCJvcGVuQWNjZXNzSWQiOiIxMzQ4NTgzNDA1NjEzOTM2NDAzIn0sInRpdGxlIjoiRWZmZWN0IG9mIG5hbm9idWJibGVzIChveHlnZW4sIG96b25lKSBvbiB0aGUgUGFjaWZpYyB3aGl0ZSBzaHJpbXAgKFBlbmFldXMgdmFubmFtZWkpLCBWaWJyaW8gcGFyYWhhZW1vbHl0aWN1cyBhbmQgd2F0ZXIgcXVhbGl0eSB1bmRlciBsYWIgY29uZGl0aW9ucyIsImFic3RyYWN0IjoiVGhpcyBzdHVkeSBhc3Nlc3NlZCB0aGUgZWZmZWN0cyBvZiBveHlnZW4gYW5kIG96b25lIG5hbm9idWJibGVzIG9uIGdpbGwgbW9ycGhvbG9neSwgd2VpZ2h0IGdhaW4sIGFuZCBtb3J0YWxpdHkgb2YgUGFjaWZpYyB3aGl0ZSBzaHJpbXAgKFBlbmFldXMgdmFubmFtZWkpLCBhcyB3ZWxsIGFzIHRoZSBsZXZlbCBvZiBWaWJyaW8gcGFyYWhhZW1vbHl0aWN1cyBhbmQgd2F0ZXIgcXVhbGl0eSBvZiBzaHJpbXAgY3VsdHVyZSB0YW5rcyB1bmRlciBsYWIgY29uZGl0aW9ucy4gVHdvIGV4cGVyaW1lbnRzIHdlcmUgY2FycmllZCBvdXQgd2l0aCBveHlnZW4gbWFjcm9idWJibGUsIG96b25lIG1hY3JvYnViYmxlLCBveHlnZW4gbmFub2J1YmJsZSwgb3pvbmUgbmFub2J1YmJsZSwgYW5kIGNvbnRyb2wgdHJlYXRtZW50cyAoYWlyLXN0b25lIG1hY3JvYnViYmxlKS4gRXhwZXJpbWVudHMgd2VyZSBkb25lIGluIHRyaXBsaWNhdGUgaW4gMTAwIEwgdGFua3Mgd2l0aCAxNeKAsCBzYWxpbmUgd2F0ZXIsIGFuZCAyMCBzaHJpbXAgcGVyIHRhbmsuIFRhbmtzIGluIEV4cGVyaW1lbnQgMSB3ZXJlIG5vdCBpbm9jdWxhdGVkIHdpdGggYmFjdGVyaWE7IHRhbmtzIGluIEV4cGVyaW1lbnQgMiB3ZXJlIGlub2N1bGF0ZWQgd2l0aCBWLiBwYXJhaGFlbW9seXRpY3VzIGF0IGEgY29uY2VudHJhdGlvbiBvZiAxMDYgQ0ZVL21MLiBUaGUgcmVzdWx0cyByZXZlYWxlZCB0aGF0IHNob3J0IHRyZWF0bWVudHMgd2l0aCBvem9uZSBuYW5vYnViYmxlcyBoYWQgbWluaW1hbCBpbXBhY3Qgb24gc2hyaW1wIGdpbGxzLCBtb3J0YWxpdHksIGFuZCBncm93dGggcmF0ZXMsIHJlZHVjZWQgVi4gcGFyYWhhZW1vbHl0aWN1cyBjb25jZW50cmF0aW9uIGluIHdhdGVyIGNvbXBhcmVkIHRvIHRoZSBvdGhlciBncm91cHMsIGFuZCBpbXByb3ZlIHdhdGVyIHF1YWxpdHkuIFRoZXNlIGxhYm9yYXRvcnkgcmVzdWx0cyBpbmRpY2F0ZSB0aGF0IG96b25lIG5hbm9idWJibGUgdHJlYXRtZW50IG1heSBiZSB1c2VmdWwgZm9yIGNvbnRyb2xsaW5nIFYuIHBhcmFoYWVtb2x5dGljdXMuIE1vcmUgd29yayBpcyBuZWVkZWQgdG8gZmluZCB0aGUgYmVzdCBwcm90b2NvbCB0byBhcHBseSB0aGUgdGVjaG5vbG9neSBvbiBhIGNvbW1lcmNpYWwgc2NhbGUuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjQ3ODUzL2Zhcy4yMDIyLmUzOSIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cuZS1mYXMub3JnL2Rvd25sb2FkL2Rvd25sb2FkX3BkZj9waWQ9ZmFzLTI1LTgtNDI5IiwiaXNzdWVkRGF0ZSI6IjIwMjItMDgtMDEiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA5IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJGaXNoZXJpZXMgYW5kIGFxdWF0aWMgc2NpZW5jZXMiLCJpc3N1ZSI6OCwidm9sdW1lIjoyNSwiaW1wYWN0RmFjdG9yIjoiMS4zNCJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjQyOSwibGFzdFBhZ2UiOjQ0MH19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJTcHJpbmdlciBTY2llbmNlK0J1c2luZXNzIE1lZGlhIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjozMCwiY2l0ZWRCeUNvdW50IjoxM319LCJxdW90ZSI6IkEgbmFub2J1YmJsZSBnZW5lcmF0b3IgbW9kZWwgYVF1YSs3NU1PIChBcXVhUHJvIFNvbHV0aW9ucywgU2luZ2Fwb3JlLCBTaW5nYXBvcmUpIHdpdGggYSAxIEhQIHBvd2VyICgwLjc1IGtXKSBodHRwczovL2RvaS5vcmcvMTAuNDc4NTMvRkFTLjIwMjIuZTM5IGh0dHBzOi8vd3d3LmUtZmFzLm9yZyB8IDQzMSBOZ3V5ZW4gSHV1IE5naGlhLCBldCBhbC4gRmlzaGVyaWVzIGFuZCBBcXVhdGljIFNjaWVuY2VzIG1vdG9yIGFuZCBhIHdhdGVyIGZsb3cgb3V0cHV0IHJhdGUgb2YgMSwwMDAgTC9oIHdhcyB1c2VkIHRvIGdlbmVyYXRlIG5hbm9idWJibGVzLiBUaGUgbWFjaGluZSBwcm9kdWNlZCBuYW5vYnViYmxlcyAgd2l0aCBhbiBhdmVyYWdlIHNpemUgb2YgMTY4LjkgwrEgNzMuOCBubSwgYW5kIGEgY29uY2VudHJhdGlvbiBvZiAgMS4wNCDDlyAxMCA5IMKxIDIuNiDDlyAxMCA4IHBhcnRpY2xlcy9tTCAoQXF1YVBybyBTb2x1dGlvbnMsIDIwMTkpLiBUbyBjcmVhdGUgb3h5Z2VuIG5hbm9idWJibGVzLCB3ZSB1c2VkIGFuIG94eWdlbiBjb25jZW50cmF0b3IgKG1vZGVsIFl1d2VsbCA3Ri0xMCwgWXV3ZWxsLCBEYW55YW5nLCBDaGluYTsgMTAgTC9taW4gbWF4ICBmbG93cmF0ZSwg4omlIDk1LjUlIG94eWdlbiBjb25jZW50cmF0aW9uLCBmbG93IHJhdGUgYXQgNCBML21pbikgIHByb3ZpZGVkIG94eWdlbiBmb3IgbmFub2J1YmJsZSBnZW5lcmF0b3IuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6ODYuMjIxNDA1MDI5Mjk2ODgsImxlZnQiOjU0LjA0NzU1MDIwMTQxNjAxNiwid2lkdGgiOjM4LjIwMTI0ODE2ODk0NTMxLCJoZWlnaHQiOjEuNjk5NjMxMzMzMzUxMTM1M319LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjg3Ljk4NTI5ODE1NjczODI4LCJsZWZ0Ijo1MS4xOTA0MDY3OTkzMTY0MDYsIndpZHRoIjo0MS4wNjAxMjM0NDM2MDM1MTYsImhlaWdodCI6MS42OTk2MzEzMzMzNTExMzUzfX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6OTQuOTg3OTM3OTI3MjQ2MSwibGVmdCI6OC4wOTUyMzU4MjQ1ODQ5NjEsIndpZHRoIjo4NC4yODU3MjA4MjUxOTUzMSwiaGVpZ2h0IjoxLjY2MTc1MDY3NDI0Nzc0MTd9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjozLjcyODQ2MTI2NTU2Mzk2NSwibGVmdCI6OC4xMDQ3NjIwNzczMzE1NDMsIndpZHRoIjoxNi4yNTg3Njk5ODkwMTM2NzIsImhlaWdodCI6MS4yNjY4MTA1MzYzODQ1ODI1fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6NC43MTkyNzIxMzY2ODgyMzIsImxlZnQiOjcwLjQzNDcyMjkwMDM5MDYyLCJ3aWR0aCI6MTQuMDMwNjkwMTkzMTc2MjcsImhlaWdodCI6MS4zMDA3NjkzMjkwNzEwNDV9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjoxMC4zNzQyMDQ2MzU2MjAxMTcsImxlZnQiOjguMDk1MjM1ODI0NTg0OTYxLCJ3aWR0aCI6NDEuMDc4NzU4MjM5NzQ2MDk0LCJoZWlnaHQiOjYuOTkxMjk4Njc1NTM3MTA5fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MTUuNTk2MjI3NjQ1ODc0MDIzLCJsZWZ0IjoxNC4xNjc3MTQxMTg5NTc1Miwid2lkdGgiOjcuNjQwNDU2MTk5NjQ1OTk2LCJoZWlnaHQiOjEuNzY5Mjc1MTg4NDQ2MDQ1fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MTUuNTk2MjI3NjQ1ODc0MDIzLCJsZWZ0IjoyMS43NzczMDk0MTc3MjQ2MSwid2lkdGgiOjI3LjM3NzUzMjk1ODk4NDM3NSwiaGVpZ2h0IjoxLjc2OTI3NTE4ODQ0NjA0NX19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjE3LjQyOTc1ODA3MTg5OTQxNCwibGVmdCI6OC4wOTUyMzU4MjQ1ODQ5NjEsIndpZHRoIjo0MS4wNjAzMDY1NDkwNzIyNjYsImhlaWdodCI6MS42OTk2MzMxMjE0OTA0Nzg1fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MTkuMTkzNjQ1NDc3Mjk0OTIyLCJsZWZ0Ijo4LjA5NTIzNTgyNDU4NDk2MSwid2lkdGgiOjQxLjA2OTg2OTk5NTExNzE5LCJoZWlnaHQiOjEuNjk5NjMzMTIxNDkwNDc4NX19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjIwLjk1NzUzNDc5MDAzOTA2MiwibGVmdCI6OC4wOTUyMzU4MjQ1ODQ5NjEsIndpZHRoIjo0MS4wNTg3NzY4NTU0Njg3NSwiaGVpZ2h0IjoxLjY5OTYzMzEyMTQ5MDQ3ODV9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjoyMi43MjE0MjQxMDI3ODMyMDMsImxlZnQiOjguMDk1MjM1ODI0NTg0OTYxLCJ3aWR0aCI6NDAuNzE0MjkwNjE4ODk2NDg0LCJoZWlnaHQiOjEuNjk5NjMzMTIxNDkwNDc4NX19XX0sImNodW5rSWQiOiJiOWVlNzkxMS1hMTI1LTQzYWUtYTRjMi0wM2RlMWZhYjYyOWUiLCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI0MzAifX1d. Beyond these operational specifications, the efficacy of such systems is often optimized by modifying geometric parameters, such as the number and diameter of holes in perforated plates, to influence factors like temperature and oxygen concentration (Zhu et al., 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5pbmciLCJmYW1pbHkiOiJaaHUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTWluLVl1IiwiZmFtaWx5IjoiTGkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiS29oZWkiLCJmYW1pbHkiOiJTaGliYXRhIn19XSwiaWRzIjp7Imlzc24iOltdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMzM5MC9lbmdwcm9jMjAyMzA1NTAyMyIsIm9wZW5hbGV4IjoiVzQzODkxNDU1NzgiLCJleGEiOiJodHRwczovL3d3dy5tZHBpLmNvbS8yNjczLTQ1OTEvNTUvMS8yMyJ9LCJ0aXRsZSI6IlRoZSBEZXZlbG9wbWVudCBvZiBhIEhpZ2gtQ29uY2VudHJhdGlvbiBPeHlnZW5hdGVkIFdhdGVyIEdlbmVyYXRvciBCYXNlZCBvbiBOYW5vYnViYmxlcyBhbmQgSXRzIEFwcGxpY2F0aW9uIiwiYWJzdHJhY3QiOiJXYXRlciB3aXRoIGEgaGlnaCBjb25jZW50cmF0aW9uIG9mIG94eWdlbiBpcyBuZWVkZWQgZm9yIHRoZSBhcXVhY3VsdHVyZSBpbmR1c3RyeSBpbiBKYXBhbi4gSW4gdGhlIGN1cnJlbnQgc3R1ZHksIHRoZSBwcmVzc3VyaXplZCBkaXNzb2x1dGlvbiBtZXRob2Qgd2FzIGVtcGxveWVkIHRvIGdlbmVyYXRlIGhpZ2gtY29uY2VudHJhdGlvbiBveHlnZW5hdGVkIHdhdGVyIChIQ09XKSBieSBwcm9kdWNpbmcgb3h5Z2VuIG5hbm9idWJibGVzIGluIHRoZSB3YXRlci4gSW4gb3JkZXIgdG8gaW52ZXN0aWdhdGUgZmFjdG9ycyBzdWNoIGFzIHRlbXBlcmF0dXJlLCBnZW9tZXRyaWMgY29uZGl0aW9ucywgYW5kIHRoZWlyIGluZmx1ZW5jZSBvbiB0aGUgb3h5Z2VuIGNvbmNlbnRyYXRpb24sIGEgc3BlY2lhbCBuYW5vYnViYmxlIGdlbmVyYXRvciB3YXMgaW1wcm92ZWQgYnkgY2hhbmdpbmcgdGhlIG51bWJlciBhbmQgdGhlIGRpYW1ldGVyIG9mIHRoZSBob2xlcyBvZiB0aGUgcGVyZm9yYXRlZCBwbGF0ZSBpbiB0aGlzIHN0dWR5LiBUaGVuLCBhbiBleHBlcmltZW50YWwgc3lzdGVtIHdoZXJlIG94eWdlbiBhbmQgd2F0ZXIgd2VyZSBzZXBhcmF0ZWx5IGludHJvZHVjZWQgaW5zaWRlIHRoZSBwcm9wb3NlZCBuYW5vYnViYmxlIGdlbmVyYXRvciB3YXMgZGVzaWduZWQuIFRoZSBkaXNzb2x2ZWQgb3h5Z2VuIGNvbmNlbnRyYXRpb24gd2FzIG1lYXN1cmVkIHVuZGVyIGRpZmZlcmVudCBjb25kaXRpb25zLiBGaW5hbGx5LCB0aGUgcHJvZHVjZWQgSENPVyB3YXMgdXNlZCB0byBjdWx0aXZhdGUgYSBtaW5pLXN1bmZsb3dlci4gVGhyb3VnaCBhIHNlcmllcyBvZiBleHBlcmltZW50cywgaXQgd2FzIGZvdW5kIHRoYXQgd2l0aCB0aGUgaW1wcm92ZWQgcGVyZm9yYXRlZCBwbGF0ZSwgdGhlIGRpc3NvbHZlZCBveHlnZW4gY29uY2VudHJhdGlvbiB3YXMgaW5jcmVhc2VkIGFuZCB0aGUgbmFub2J1YmJsZSBnZW5lcmF0b3IgcmVhY2hlZCB0aGUgc2F0dXJhdGlvbiBzdGF0ZSBxdWlja2x5LCB3aGlsZSB0aGUgbWluaS1zdW5mbG93ZXIgY3VsdGl2YXRlZCB3aXRoIHRoZSBIQ09XIGFwcGVhcmVkIHRvIGdyb3cgbGFyZ2VyIHRoYW4gdGhhdCB3aXRoIHRhcCB3YXRlci4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMzM5MC9lbmdwcm9jMjAyMzA1NTAyMyIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cubWRwaS5jb20vMjY3My00NTkxLzU1LzEvMjMvcGRmP3ZlcnNpb249MTcwMTI0MzExOCIsImlzc3VlZERhdGUiOiIyMDIzLTExLTI5IiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInZvbHVtZSI6MzN9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjIzLCJsYXN0UGFnZSI6MjN9fSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjIsImNpdGVkQnlDb3VudCI6MX19LCJxdW90ZSI6IkFsdG1ldHJpY1tfc2hhcmVfIFNoYXJlXShodHRwczovL3d3dy5tZHBpLmNvbS8yNjczLTQ1OTEvNTUvMS8yMykgW19hbm5vdW5jZW1lbnRfIEhlbHBdKGh0dHBzOi8vd3d3Lm1kcGkuY29tLzI2NzMtNDU5MS81NS8xLzIzKSBbX2Zvcm1hdFxcX3F1b3RlXyBDaXRlXShqYXZhc2NyaXB0OnZvaWQoMCk7KSBbX3F1ZXN0aW9uXFxfYW5zd2VyXyBEaXNjdXNzIGluIFNjaVByb2ZpbGVzXShodHRwczovL3NjaXByb2ZpbGVzLmNvbS9kaXNjdXNzaW9uLWdyb3Vwcy9wdWJsaWMvMTAuMzM5MC9lbmdwcm9jMjAyMzA1NTAyMz91dG1fc291cmNlPW1wZGkuY29tJnV0bV9tZWRpdW09cHVibGljYXRpb24mdXRtX2NhbXBhaWduPWRpc2N1c3NfaW5fc2NpcHJvZmlsZXMpIFdhdGVyIHdpdGggYSBoaWdoIGNvbmNlbnRyYXRpb24gb2Ygb3h5Z2VuIGlzIG5lZWRlZCBmb3IgdGhlIGFxdWFjdWx0dXJlIGluZHVzdHJ5IGluIEphcGFuLiBJbiB0aGUgY3VycmVudCBzdHVkeSwgdGhlIHByZXNzdXJpemVkIGRpc3NvbHV0aW9uIG1ldGhvZCB3YXMgZW1wbG95ZWQgdG8gZ2VuZXJhdGUgaGlnaC1jb25jZW50cmF0aW9uIG94eWdlbmF0ZWQgd2F0ZXIgKEhDT1cpIGJ5IHByb2R1Y2luZyBveHlnZW4gbmFub2J1YmJsZXMgaW4gdGhlIHdhdGVyLiBJbiBvcmRlciB0byBpbnZlc3RpZ2F0ZSBmYWN0b3JzIHN1Y2ggYXMgdGVtcGVyYXR1cmUsIGdlb21ldHJpYyBjb25kaXRpb25zLCBhbmQgdGhlaXIgaW5mbHVlbmNlIG9uIHRoZSBveHlnZW4gY29uY2VudHJhdGlvbiwgYSBzcGVjaWFsIG5hbm9idWJibGUgZ2VuZXJhdG9yIHdhcyBpbXByb3ZlZCBieSBjaGFuZ2luZyB0aGUgbnVtYmVyIGFuZCB0aGUgZGlhbWV0ZXIgb2YgdGhlIGhvbGVzIG9mIHRoZSBwZXJmb3JhdGVkIHBsYXRlIGluIHRoaXMgc3R1ZHkuIiwiZXh0ZXJuYWxDYWNoZUlkIjoiNjYxNGFlMzAtNDE4OC0zNDBmLTliMDUtYjljMmJiYjE0ZjdiIn1d.

2.2. Properties and Mechanisms of Action

Nanobubbles, defined as gas-filled cavities with diameters typically below 1 μm (often 100-200 nm), exhibit unique physicochemical properties that distinguish them from larger microbubbles and enable superior performance in gas dissolution, mass transfer, and chemical reactivity (Nghĩa et al., 2022)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5ndXnhu4VuIEjhu691IiwiZmFtaWx5IjoiTmdoxKlhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5ndXllbiBUaGkiLCJmYW1pbHkiOiJOZ3V5ZW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUGhhbiBUcuG7jW5nIiwiZmFtaWx5IjoiQsOsbmgifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTGUgVGhpIiwiZmFtaWx5IjoiTWF5In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRvbmcgVHJhbiIsImZhbWlseSI6Ikh1eSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQaOG6oW0gVGjDoWkiLCJmYW1pbHkiOiJHaWFuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTb3BoaWUiLCJmYW1pbHkiOiJTdOKAkEhpbGFpcmUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUGhhbiBUaGkiLCJmYW1pbHkiOiJWYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMjM0LTE3NDkiLCIyMjM0LTE3NTciXSwiaXNibiI6W10sImRvaSI6IjEwLjQ3ODUzL2Zhcy4yMDIyLmUzOSIsIm9wZW5hbGV4IjoiVzQyOTQ1OTc1NTQifSwidGl0bGUiOiJFZmZlY3Qgb2YgbmFub2J1YmJsZXMgKG94eWdlbiwgb3pvbmUpIG9uIHRoZSBQYWNpZmljIHdoaXRlIHNocmltcCAoUGVuYWV1cyB2YW5uYW1laSksIFZpYnJpbyBwYXJhaGFlbW9seXRpY3VzIGFuZCB3YXRlciBxdWFsaXR5IHVuZGVyIGxhYiBjb25kaXRpb25zIiwiYWJzdHJhY3QiOiJUaGlzIHN0dWR5IGFzc2Vzc2VkIHRoZSBlZmZlY3RzIG9mIG94eWdlbiBhbmQgb3pvbmUgbmFub2J1YmJsZXMgb24gZ2lsbCBtb3JwaG9sb2d5LCB3ZWlnaHQgZ2FpbiwgYW5kIG1vcnRhbGl0eSBvZiBQYWNpZmljIHdoaXRlIHNocmltcCAoUGVuYWV1cyB2YW5uYW1laSksIGFzIHdlbGwgYXMgdGhlIGxldmVsIG9mIFZpYnJpbyBwYXJhaGFlbW9seXRpY3VzIGFuZCB3YXRlciBxdWFsaXR5IG9mIHNocmltcCBjdWx0dXJlIHRhbmtzIHVuZGVyIGxhYiBjb25kaXRpb25zLiBUd28gZXhwZXJpbWVudHMgd2VyZSBjYXJyaWVkIG91dCB3aXRoIG94eWdlbiBtYWNyb2J1YmJsZSwgb3pvbmUgbWFjcm9idWJibGUsIG94eWdlbiBuYW5vYnViYmxlLCBvem9uZSBuYW5vYnViYmxlLCBhbmQgY29udHJvbCB0cmVhdG1lbnRzIChhaXItc3RvbmUgbWFjcm9idWJibGUpLiBFeHBlcmltZW50cyB3ZXJlIGRvbmUgaW4gdHJpcGxpY2F0ZSBpbiAxMDAgTCB0YW5rcyB3aXRoIDE14oCwIHNhbGluZSB3YXRlciwgYW5kIDIwIHNocmltcCBwZXIgdGFuay4gVGFua3MgaW4gRXhwZXJpbWVudCAxIHdlcmUgbm90IGlub2N1bGF0ZWQgd2l0aCBiYWN0ZXJpYTsgdGFua3MgaW4gRXhwZXJpbWVudCAyIHdlcmUgaW5vY3VsYXRlZCB3aXRoIFYuIHBhcmFoYWVtb2x5dGljdXMgYXQgYSBjb25jZW50cmF0aW9uIG9mIDEwNiBDRlUvbUwuIFRoZSByZXN1bHRzIHJldmVhbGVkIHRoYXQgc2hvcnQgdHJlYXRtZW50cyB3aXRoIG96b25lIG5hbm9idWJibGVzIGhhZCBtaW5pbWFsIGltcGFjdCBvbiBzaHJpbXAgZ2lsbHMsIG1vcnRhbGl0eSwgYW5kIGdyb3d0aCByYXRlcywgcmVkdWNlZCBWLiBwYXJhaGFlbW9seXRpY3VzIGNvbmNlbnRyYXRpb24gaW4gd2F0ZXIgY29tcGFyZWQgdG8gdGhlIG90aGVyIGdyb3VwcywgYW5kIGltcHJvdmUgd2F0ZXIgcXVhbGl0eS4gVGhlc2UgbGFib3JhdG9yeSByZXN1bHRzIGluZGljYXRlIHRoYXQgb3pvbmUgbmFub2J1YmJsZSB0cmVhdG1lbnQgbWF5IGJlIHVzZWZ1bCBmb3IgY29udHJvbGxpbmcgVi4gcGFyYWhhZW1vbHl0aWN1cy4gTW9yZSB3b3JrIGlzIG5lZWRlZCB0byBmaW5kIHRoZSBiZXN0IHByb3RvY29sIHRvIGFwcGx5IHRoZSB0ZWNobm9sb2d5IG9uIGEgY29tbWVyY2lhbCBzY2FsZS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuNDc4NTMvZmFzLjIwMjIuZTM5IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5lLWZhcy5vcmcvZG93bmxvYWQvZG93bmxvYWRfcGRmP3BpZD1mYXMtMjUtOC00MjkiLCJpc3N1ZWREYXRlIjoiMjAyMi0wOC0wMSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZpc2hlcmllcyBhbmQgYXF1YXRpYyBzY2llbmNlcyIsImlzc3VlIjo4LCJ2b2x1bWUiOjI1LCJpbXBhY3RGYWN0b3IiOiIxLjM0In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6NDI5LCJsYXN0UGFnZSI6NDQwfX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlNwcmluZ2VyIFNjaWVuY2UrQnVzaW5lc3MgTWVkaWEifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjMwLCJjaXRlZEJ5Q291bnQiOjEzfX0sInF1b3RlIjoiVGhlIGluZHVzdHJ5IHRoZXJlZm9yZSBuZWVkcyB0byBmaW5kIGFsdGVybmF0aXZlIG1lYXN1cmVzICB0byBwcmV2ZW50IGFuZCBjb250cm9sIHRoaXMgcGF0aG9nZW4uIE5hbm9idWJibGUgdGVjaG5vbG9neSBwcm9kdWNlcyBzbWFsbCBnYXMgYnViYmxlcyB3aXRoIGEgZGlhbWV0ZXIgb2YgbGVzcyB0aGFuIDIwMCBubSAoQWdyYXdhbCBldCBhbC4sIDIwMTE7IENoYXBsaW4sIDIwMjEpIHdoaWNoIGNhbiBleGlzdCBpbiB0aGUgd2F0ZXIgY29sdW1uIGZvciB3ZWVrcyAoQXpldmVkbyAgZXQgYWwuLCAyMDE2OyBQYXJtYXIgJiBNYWp1bWRlciwgMjAxMykuIFRoaXMgcGhlbm9tZW5vbiBpcyAgYXNzb2NpYXRlZCB3aXRoIHRoZSBwaHlzaWNhbCBjaGFyYWN0ZXJpc3RpYyBvZiB0aGVzZSBzbWFsbCBidWItYmxlcyAoV2VpanMgJiBMb2hzZSwgMjAxMykgYW5kIGhpZ2ggemV0YSBwb3RlbnRpYWwgdmFsdWUgd2hpY2ggIGtlZXBzIHRoZW0gZnJvbSBhZ2dyZWdhdGluZyBpbnRvIGxhcmdlciBidWJibGVzIChLaXJieSwgMjAxMDsgIFNqb2dyZWVuIGV0IGFsLiwgMjAxOCkuIE5hbm9idWJibGUgdGVjaG5vbG9neSBoYXMgYSBsb3Qgb2YgcHJvbS1pc2luZyBlZmZlY3RpdmUgYXBwbGljYXRpb24gaW4gYXF1YWN1bHR1cmUgYmVjYXVzZSBvZiBpdHMgYWJpbGl0eSAgdG8gaW5jcmVhc2UgYW5kIG1haW50YWluIG94eWdlbiBjb250ZW50IGluIHdhdGVyIGFuZCBjb250cm9sICBiYWN0ZXJpYSwgYSBwb3NzaWJsZSBtZWFzdXJlIHRvIHJlZHVjZSB0aGUgdXNlIG9mIGFudGliaW90aWNzLiBPeHlnZW4gbmFub2J1YmJsZXMgb3B0aW1pemUgdGhlIGRpc3NvbHZlZCBveHlnZW4gaW4gYXF1YXRpYyBzeXN0ZW1zIChHYWxhbmcgZXQgYWwuLCAyMDE5KS4gSGlnaCBsZXZlbHMgb2Ygb3h5Z2VuIG1heSBzcGVlZCB1cCBtZXRhYm9saXNtLCBwcm9tb3RlIGhpZ2hlciBmb29kIGludGFrZSwgYW5kIGluY3JlYXNlICBncm93dGggb2YgZmFybWVkIHNocmltcCAoRWJpbmEgZXQgYWwuLCAyMDEzKS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNS4wNjg2NDU0NzcyOTQ5MiwibGVmdCI6OC4wOTUyMzU4MjQ1ODQ5NjEsIndpZHRoIjo0MS4wNzIyMTIyMTkyMzgyOCwiaGVpZ2h0IjoxLjY5OTYzNTE0ODA0ODQwMDl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNi44MzI1MzA5NzUzNDE4LCJsZWZ0Ijo4LjA5NTIzNTgyNDU4NDk2MSwid2lkdGgiOjIzLjYwNDE4NTEwNDM3MDExNywiaGVpZ2h0IjoxLjY5OTYzNTE0ODA0ODQwMDl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC41OTY0MjAyODgwODU5NCwibGVmdCI6MTAuOTUyMzY0OTIxNTY5ODI0LCJ3aWR0aCI6MzguMjE1ODc3NTMyOTU4OTg0LCJoZWlnaHQiOjEuNjk5NjM1MTQ4MDQ4NDAwOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQwLjM2MDMwOTYwMDgzMDA4LCJsZWZ0Ijo4LjA5NTIzNTgyNDU4NDk2MSwid2lkdGgiOjQxLjA3Njc1OTMzODM3ODkwNiwiaGVpZ2h0IjoxNy41NzQ2MzA3MzczMDQ2ODh9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1Ny45OTkxOTUwOTg4NzY5NSwibGVmdCI6MTAuOTUyMzY0OTIxNTY5ODI0LCJ3aWR0aCI6MzguMjAxNDQ2NTMzMjAzMTI1LCJoZWlnaHQiOjEuNjk5NjM1MTQ4MDQ4NDAwOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjU5Ljc2MzA4MDU5NjkyMzgzLCJsZWZ0Ijo4LjA5NTIzNTgyNDU4NDk2MSwid2lkdGgiOjQxLjA2MzMyNzc4OTMwNjY0LCJoZWlnaHQiOjEuNjk5NjM1MTQ4MDQ4NDAwOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjYxLjUyNjk3MzcyNDM2NTIzNCwibGVmdCI6OC4wOTUyMzU4MjQ1ODQ5NjEsIndpZHRoIjo0MS4wNjY4NDQ5NDAxODU1NSwiaGVpZ2h0IjoxLjY5OTYzNTE0ODA0ODQwMDl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2My4yOTA4NjMwMzcxMDkzNzUsImxlZnQiOjguMDk1MjM1ODI0NTg0OTYxLCJ3aWR0aCI6NDAuNzE0MjkwNjE4ODk2NDg0LCJoZWlnaHQiOjEuNjk5NjMxMzMzMzUxMTM1M319XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjQzMCJ9fV0%3D. Their exceptionally high surface area-to-volume ratio-scaling inversely with radius-facilitates rapid diffusion of gases like oxygen or ozone into the surrounding liquid, achieving oxygen transfer efficiencies up to several times higher than conventional aeration methods. This is particularly advantageous in dynamic environments such as aquaculture, where maintaining supersaturated dissolved oxygen levels is critical for aquatic organism health. Nanobubbles demonstrate remarkable metastability, persisting in water for days to months, defying classical Epstein-Plesset dissolution theory due to specialized stabilization mechanisms (Svetovoy, 2021)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlYuIEIuIiwiZmFtaWx5IjoiU3ZldG92b3kifX1dLCJpZHMiOnsiaXNzbiI6WyIxMzU5LTAyOTQiLCIxODc5LTAzOTkiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMTYvai5jb2Npcy4yMDIxLjEwMTQyMyIsIm1hZyI6IjMxMjY0ODQ4NDgiLCJvcGVuYWxleCI6IlczMTI2NDg0ODQ4In0sInRpdGxlIjoiU3BvbnRhbmVvdXMgY2hlbWljYWwgcmVhY3Rpb25zIGJldHdlZW4gaHlkcm9nZW4gYW5kIG94eWdlbiBpbiBuYW5vYnViYmxlcyIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2ouY29jaXMuMjAyMS4xMDE0MjMiLCJyZXNvdXJjZVVybCI6Imh0dHA6Ly9hcnhpdi5vcmcvcGRmLzIxMDIuMDMxMjYiLCJpc3N1ZWREYXRlIjoiMjAyMS0wMi0wNSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkN1cnJlbnQgT3BpbmlvbiBpbiBDb2xsb2lkICYgSW50ZXJmYWNlIFNjaWVuY2UiLCJ2b2x1bWUiOjUyfSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjoxMDE0MjMsImxhc3RQYWdlIjoxMDE0MjN9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRWxzZXZpZXIgQlYifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjYxLCJjaXRlZEJ5Q291bnQiOjJ9fSwicXVvdGUiOiJUaGlzIGF0dGVudGlvbiBpcyBkcml2ZW4gYnkgYSBmdW5kYS1tZW50YWwgcHJvYmxlbTogTkJzIGxpdmUgbXVjaCB0b28gbG9uZyBpbiBjb21wYXJpLXNvbiB3aXRoIHRoZSBwcmVkaWN0aW9uIGdpdmVuIGJ5IHRoZSBkaWZmdXNpb24gZGlzc29sdS10aW9uIHRoZW9yeSBbNCwgNV0uIE9uIHRoZSBvdGhlciBoYW5kLCBleGNpdGluZyBhcHBsaWNhLXRpb25zIG9mIE5CcyBpbiBlbnZpcm9ubWVudCBbNiwgN10sIGFncmljdWx0dXJlIFs4XSwgZmxvdGEtdGlvbiBbOSwgMTBdLCBoZWFsdGhjYXJlIFsxMSwgMTJdLCBhbmQgY2hlbWlzdHJ5IFsxM10gaGF2ZSBlbWVyZ2VkIHRvIG5hbWUgb25seSBhIGZldy4gSW4gY29udHJhc3Qgd2l0aCBzdXJmYWNlIE5Ccywgd2hpY2ggZXhpc3Qgb24gdGhlIHNvbGlkLWxpcXVpZCBpbnRlcmZhY2UsIGJ1bGsgTkJzIGZpbGwgdGhlIHZvbHVtZSBvZiBsaXF1aWQuIFRoZXNlIE5CcyBhcmUgZ2VuZXJhdGVkIGJ5IG1vc3RseSBtZWNoYW5pY2FsIGluamVjdGlvbiBvZiBkaWYtZmVyZW50IGdhc2VzIFs5XSBvciBieSB1bHRyYXNvbmljIGNhdml0YXRpb24gWzEwXSwgYnV0IGh5LWRyb2dlbiBhbmQgb3h5Z2VuIE5CcyB3aGVyZSBhbHNvIGdlbmVyYXRlZCBieSBEQyBlbGVjdHJvbHlzaXMgWzE0LCAxNV0uIExhdGVyIHRoZXNlIG1ldGhvZHMgd2lsbCBiZSBjYWxsZWQgdGhlIHJlZ3VsYXIgbWV0aG9kcy4gVGhlIHNpemUgZGlzdHJpYnV0aW9uIG9mIHRoZSBOQnMgd2FzIGNoYXJhY3Rlcml6ZWQgYnkgdGhlIGR5bmFtaWMgbGlnaHQgc2NhdHRlcmluZyBhbmQgaW5kaXZpZHVhbCBidWJibGVzIHdlcmUgb2JzZXJ2ZWQgYnkgZWxlY3Ryb24gbWljcm9zY29weSBmcm9tIGZyZWV6ZSBmcmFjdHVyZSByZXBsaWNhcyBbMTZdLCBieSBwaGFzZSBtaWNyb3Njb3B5IGFuZCBwb2xhcmltZXRyaWMgc2NhdHRlcm9tZXRyeSBbMTddLiBVbmV4cGVjdGVkbHksIE5CcyBjb250YWluaW5nIG94eWdlbiBhcmUgYWJsZSB0byBwcm8tZHVjZSByZWFjdGl2ZSBveHlnZW4gc3BlY2llcyB3aGljaCBjYW4gb3hpZGl6ZSBwb2xsdS10YW50cyBhbmQgcGF0aG9nZW5zIGluIHdhdGVyIFsxOOKAkzIwXSwgd2hpbGUgc2VwYXJhdGUgb3h5LWdlbiBtb2xlY3VsZXMgaGF2ZSBubyBlZmZlY3Qgb24gb3JnYW5pY3MuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6MzYuOTUwNjQ1NDQ2Nzc3MzQ0LCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA1MjI2MTM1MjUzOTA2LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjM4LjM5NzIzNTg3MDM2MTMzLCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA0ODk5OTc4NjM3Njk1LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjM5Ljg0MzgyNjI5Mzk0NTMxLCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA1MjI2MTM1MjUzOTA2LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjQxLjI5MDQxNjcxNzUyOTMsImxlZnQiOjguODIzNTI1NDI4NzcxOTczLCJ3aWR0aCI6NDAuMDUzODgyNTk4ODc2OTUsImhlaWdodCI6MS4yNTc5MDU5NjAwODMwMDc4fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NDIuNzM3MDEwOTU1ODEwNTUsImxlZnQiOjguODIzNTI1NDI4NzcxOTczLCJ3aWR0aCI6NDAuMDUyMjQ2MDkzNzUsImhlaWdodCI6MS4yNTc5MDU5NjAwODMwMDc4fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NDQuMTgzNjAxMzc5Mzk0NTMsImxlZnQiOjguODIzNTI1NDI4NzcxOTczLCJ3aWR0aCI6NDAuMDUyMjQ2MDkzNzUsImhlaWdodCI6MS4yNTc5MDU5NjAwODMwMDc4fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NDUuNjMwMTg3OTg4MjgxMjUsImxlZnQiOjguODIzNTI1NDI4NzcxOTczLCJ3aWR0aCI6MjAuNTMyMzk4MjIzODc2OTUzLCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjQ3LjA3Njc4MjIyNjU2MjUsImxlZnQiOjEwLjQ1MTQ2NjU2MDM2Mzc3LCJ3aWR0aCI6MzguNDE5NDk4NDQzNjAzNTE2LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjQ4LjUyMzM3MjY1MDE0NjQ4NCwibGVmdCI6OC44MjM1MjU0Mjg3NzE5NzMsIndpZHRoIjo0MC4wNTM4ODY0MTM1NzQyMiwiaGVpZ2h0IjoxLjI1NzkwNTk2MDA4MzAwNzh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo0OS45Njk5NjY4ODg0Mjc3MzQsImxlZnQiOjguODIzNTI1NDI4NzcxOTczLCJ3aWR0aCI6NDAuMDQ3Mzc0NzI1MzQxOCwiaGVpZ2h0IjoxLjI1NzkwNTk2MDA4MzAwNzh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo1MS40MTY1NTczMTIwMTE3MiwibGVmdCI6OC44MjM1MjU0Mjg3NzE5NzMsIndpZHRoIjo0MC4wNTIyNDYwOTM3NSwiaGVpZ2h0IjoxLjI1NzkwNTk2MDA4MzAwNzh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo1Mi44NjMxNDM5MjA4OTg0NCwibGVmdCI6OC44MjM1MjU0Mjg3NzE5NzMsIndpZHRoIjo0MC4wNDczNzQ3MjUzNDE4LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjU0LjMwOTczODE1OTE3OTY5LCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA0NDExMzE1OTE3OTY5LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjU1Ljc1NjMzMjM5NzQ2MDk0LCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA0OTAxMTIzMDQ2ODc1LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjU3LjIwMjkxOTAwNjM0NzY1NiwibGVmdCI6OC44MjM1MjU0Mjg3NzE5NzMsIndpZHRoIjo0MC4wNDczNzQ3MjUzNDE4LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjU4LjY0OTUwOTQyOTkzMTY0LCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA0OTAwMzYwMTA3NDIyLCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjYwLjA5NjEwMzY2ODIxMjg5LCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA0NTc0NTg0OTYwOTM3NSwiaGVpZ2h0IjoxLjI1NzkwNTk2MDA4MzAwNzh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2MS41NDI2OTAyNzcwOTk2MSwibGVmdCI6OC44MjM1MjU0Mjg3NzE5NzMsIndpZHRoIjoyNS43MTIzMDMxNjE2MjEwOTQsImhlaWdodCI6MS4yNTc5MDU5NjAwODMwMDc4fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NjIuOTg5NDA2NTg1NjkzMzYsImxlZnQiOjEwLjQ1MTQ2NjU2MDM2Mzc3LCJ3aWR0aCI6MzguNDIxMTEyMDYwNTQ2ODc1LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjY0LjQzNTk5NzAwOTI3NzM0LCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA0NTc0NTg0OTYwOTM3NSwiaGVpZ2h0IjoxLjI1NzkwNTk2MDA4MzAwNzh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2NS44ODI1OTEyNDc1NTg2LCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA0ODk5NTk3MTY3OTY5LCJoZWlnaHQiOjEuMjU3OTAyMTQ1Mzg1NzQyMn19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjY3LjMyOTE5MzExNTIzNDM4LCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjMwLjY2OTE3OTkxNjM4MTgzNiwiaGVpZ2h0IjoxLjI1NzkwMjE0NTM4NTc0MjJ9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIxMDE0MjMifX1d.

A primary contributor to nanobubble longevity is their negatively charged surface, manifested as a high absolute zeta potential (often -20 to -50 mV), which generates electrostatic repulsion between bubbles, inhibiting coalescence and Ostwald ripening (Jia et al., 2023; Tekile et al., 2016)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFuZGluZXQgS2ViZWRlIiwiZmFtaWx5IjoiVGVraWxlIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikls4oCQSG8iLCJmYW1pbHkiOiJLaW0ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSmFpeWVvcCIsImZhbWlseSI6IkxlZSJ9fV0sImlkcyI6eyJpc3NuIjpbIjEyMjYtMTAyNSIsIjIwMDUtOTY4WCJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuNDQ5MS9lZXIuMjAxNi4wMjgiLCJtYWciOiIyNTIzNjY5NjU3Iiwib3BlbmFsZXgiOiJXMjUyMzY2OTY1NyJ9LCJ0aXRsZSI6IkV4dGVudCBhbmQgcGVyc2lzdGVuY2Ugb2YgZGlzc29sdmVkIG94eWdlbiBlbmhhbmNlbWVudCB1c2luZyBuYW5vYnViYmxlcyIsImFic3RyYWN0IjoiSW4gdGhpcyBzdHVkeSwgY2hhbmdlIGluIHdhdGVyLWRpc3NvbHZlZCBveHlnZW4gKERPKSB3YXMgYW5hbHl6ZWQgdW5kZXIgdmFyaW91cyBzeW50aGV0aWMgd2F0ZXIgcXVhbGl0aWVzIGFuZCBuYW5vYnViYmxlcyAoTkJzKSBhcHBsaWNhdGlvbiBjb25kaXRpb25zLCBzdWNoIGFzIGdhcyB0eXBlLCBpbml0aWFsIERPIGFzIHdlbGwgYXMgd2F0ZXIgZGlzc29sdmVkLCBzdXNwZW5kZWQgYW5kIG9yZ2FuaWMgbWF0dGVycyBjb250ZW50cy4gV2hlbiBveHlnZW4sIHJhdGhlciB0aGFuIGFpciwgd2FzIGludHJvZHVjZWQgaW50byBuaXRyb2dlbi1kZXNvcmJlZCB1bHRyYS1wdXJlIHdhdGVyLCB0aGUgc3RhZ25hdGlvbiB0aW1lIHdhcyBzaWduaWZpY2FudGx5IGluY3JlYXNlZC4gSXQgdG9vayB0ZW4gZGF5cyBmb3IgRE8gY29uY2VudHJhdGlvbiB0byBkcm9wIGJhY2sgdG8gc2F0dXJhdGlvbi4gVGhlIGhpZ2hlciB0aGUgaW5pdGlhbCBETyBjb25jZW50cmF0aW9uLCB0aGUgbG9uZ2VyIHBhcnRpY2xlcyB3ZXJlIG9ic2VydmVkIGFib3ZlIHNhdHVyYXRpb24gZHVlIHRvIHBhcnRpY2xlIHN0YWJpbGl0eSBpbXByb3ZlbWVudC4gVGhlIG94eWdlbiBtYXNzIHRyYW5zZmVyIHJhdGUgb2YgMC4wNDgyIG1nL0wvbWluIHdhcyBmb3VuZCB0byByZWFjaCBhIG1heGltdW0gYXQgYW4gZWxlY3Ryb2x5dGljIGNvbmNlbnRyYXRpb24gb2YgMC43NSBnL0wsIGJleW9uZCB3aGljaCB0aGUgdHJhbnNmZXIgcmF0ZSBkZWNyZWFzZWQgZHVlIHRvIGFkc29ycHRpb24gb2YgbmVnYXRpdmUgaW9ucyBvZiB0aGUgZWxlY3Ryb2x5dGUgYXQgdGhlIGludGVyZmFjZS4gSGlnaCBsZXZlbHMgb2YgdHVyYmlkaXR5IGNhdXNlZCBieSBzdXNwZW5kZWQgc29saWRzIGhhdmUgYmVjb21lIGEgYmFycmllciB0byBkaXNzb2x1dGlvbiBvZiBOQnMgb3h5Z2VuIGludG8gdGhlIHdhdGVyIHNvbHV0aW9uLCBhbmQgdGh1cyBhZmZlY3RlZCB0aGUgdHJhbnNmZXIgcGVyZm9ybWFuY2UuIE9uIHRoZSBvdGhlciBoYW5kLCBieSBhcHBseWluZyBOQnMgZm9yIGp1c3QgYW4gaG91ciwgdXAgdG8gNy4yJSBkZWdyYWRhdGlvbiBvZiBnbHVjb3NlIGFzIHJlcHJlc2VudGF0aXZlIG9yZ2FuaWMgbWF0dGVyIHdhcyBhY2hpZXZlZC4gVGh1cywgTkJzIHRlY2hub2xvZ3kgd291bGQgbWFpbnRhaW4gYSBoaWdoIERPIGV4dGVudCBmb3IgYW4gZXh0ZW5kZWQgZHVyYXRpb24sIGFuZCB0aHVzIGNhbiBpbXByb3ZlIHdhdGVyIHF1YWxpdHkgcHJvdmlkZWQgdGhhdCB3YXRlciBjaGVtaXN0cnkgaXMgY2xvc2VseSBtb25pdG9yZWQgZHVyaW5nIGl0cyBhcHBsaWNhdGlvbi4gS2V5d29yZHM6IEFwcGxpY2F0aW9uIGNvbmRpdGlvbnMsIERpc3NvbHZlZCBveHlnZW4sIE5hbm9idWJibGVzLCBXYXRlciBxdWFsaXR5IiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjQ0OTEvZWVyLjIwMTYuMDI4IiwicmVzb3VyY2VVcmwiOiJodHRwOi8vZWVlci5vcmcvdXBsb2FkL2Vlci0yMS00LTQyNy5wZGYiLCJpc3N1ZWREYXRlIjoiMjAxNi0wOS0xMiIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkVudmlyb25tZW50YWwgRW5naW5lZXJpbmcgUmVzZWFyY2giLCJpc3N1ZSI6NCwidm9sdW1lIjoyMSwiaW1wYWN0RmFjdG9yIjoiNC4wNSJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjQyNywibGFzdFBhZ2UiOjQzNX19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJLb3JlYW4gU29jaWV0eSBvZiBFbnZpcm9ubWVudGFsIEVuZ2luZWVyaW5nIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjoyNCwiY2l0ZWRCeUNvdW50IjozM319LCJxdW90ZSI6IlRoaXMgdmVyaWZpZXMgdGhlIHBvc3NpYmlsaXR5IG9mIHRoZSBleGlzdGVuY2UgIG9mIE5Ccywgc2luY2UgYXQgYSBoaWdoZXIgRE8gY29uY2VudHJhdGlvbiwgdGhlIG94eWdlbiBkaWZmdXNpb24gIHdvdWxkIHRha2UgbW9yZSB0aW1lLiBUaGUgaGlnaCBnYXMgY29uY2VudHJhdGlvbiBpbiB0aGUgbGlxdWlkICBleHBsYWlucyB0aGUgZXh0ZW5zaW9uIG9mIE5CIHN0YWJpbGl0eS4gIFRoZSBtYWduaXR1ZGVzIG9mIHpldGEgcG90ZW50aWFsIG9mIG94eWdlbiBuYW5vLWJ1YmJsZWQgd2F0ZXIgIHdlcmUgcG9zc2libHkgaGlnaGVyIHRoYW4gdGhvc2Ugb2YgYWlyIG5hbm8tYnViYmxlZCB3YXRlci4gQXQgIGEgaGlnaCBtYWduaXR1ZGUgemV0YSBwb3RlbnRpYWwgdmFsdWUsIHRoZSBlbGVjdHJpY2FsbHkgY2hhcmdlZCAgcGFydGljbGVzIHRlbmQgdG8gcmVwZWwgZWFjaCBvdGhlciwgYXZvaWRpbmcgYWdncmVnYXRpb24uIEZvciBidWJibGUgIGRpc3BlcnNpb24sIHRoZSBoaWdoIHpldGEgcG90ZW50aWFsIGNvdWxkIGNyZWF0ZSByZXB1bHNpb24gZm9yY2VzICB0aGF0IHdvdWxkIGF2b2lkIHRoZSBjb2FsZXNjZW5jZSBvZiBidWJibGVzLCB0aHVzIGNvbnRyaWJ1dGluZyAgdG8gdGhlIHN0YWJpbGl6YXRpb24gb2YgdGhlIGJ1YmJsZXMuIFRoZXJlZm9yZSwgdGhlIG94eWdlbiBOQnMgIGNvdWxkIGJlIHN0YWJpbGl6ZWQgaW4gd2F0ZXIgYnkgdGhlIGVsZWN0cmljYWxseSBjaGFyZ2VkIHN1cmZhY2VzLCAgd2hpbGUgdGhlIG5hbm8tYnViYmxlZCBhaXIgd291bGQgYmUgYmVsb3cgdGhlIGxpbWl0IG9mIHRoZSBzdGFiaWxpemEtdGlvbiBieSByZXB1bHNpb24gZm9yY2VzLiBGdXJ0aGVybW9yZSwgdGhlIGJpZyBkaWZmZXJlbmNlIGJldHdlZW4gdGhlIHBlcmlvZHMgaW4gd2hpY2ggIGJ1YmJsZXMgY291bGQgYmUgb2JzZXJ2ZWQgYXMgc3RhYmxlIGluIHRoZSBveHlnZW4gbmFuby1idWJibGVkICB3YXRlciAoMTAtMTUgZCkgYW5kIGluIHRoYXQgb2YgYWlyIG5hbm8tYnViYmxlZCAoMy01IGQpIGlzIGxpa2VseSBGaWcuIDYuIEdyYXBocyBzaG93aW5nIHN0YWduYXRpb24gdGltZSBmb3IgZGlmZmVyZW50IGluaXRpYWwgZGlzc29sdmVkIG94eWdlbiAoRE8pLiByZWxhdGVkIHRvIHRoZSBsb3dlciBkaXNzb2x1dGlvbiBvZiB0aGUgYWlyIGluIHdhdGVyIGluIHJlbGF0aW9uIHRvIHRoZSBkaXNzb2x1dGlvbiBvZiBveHlnZW4uICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6NjYuMTc4ODAyNDkwMjM0MzgsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6NDAuNzE0ODg5NTI2MzY3MTksImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo2Ny41NzY3NzQ1OTcxNjc5NywibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42ODgyNzA1Njg4NDc2NTYsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo2OC45NjcyMzE3NTA0ODgyOCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NTcxODA3ODYxMzI4MSwiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjcwLjM1NzY4ODkwMzgwODYsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6MjUuMzQxMjU1MTg3OTg4MjgsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3MS43NTU2NTMzODEzNDc2NiwibGVmdCI6OS41NDk2NDE2MDkxOTE4OTUsIndpZHRoIjozOS4wMjQ3ODQwODgxMzQ3NjYsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3My4xNDYxMDI5MDUyNzM0NCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NTQwMjk4NDYxOTE0MDYsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3NC41MzY1NjAwNTg1OTM3NSwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NTM0NjUyNzA5OTYwOTQsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3NS45MzQ1MzIxNjU1MjczNCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NzMyMzY4NDY5MjM4MywiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjc3LjMyNDk4OTMxODg0NzY2LCJsZWZ0Ijo3Ljg2NTU0NTI3MjgyNzE0OCwid2lkdGgiOjQwLjY0OTM1MzAyNzM0Mzc1LCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6NzguNzE1NDQ2NDcyMTY3OTcsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6NDAuNjU0NjI0OTM4OTY0ODQ0LCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODAuMTEzNDEwOTQ5NzA3MDMsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6NDAuNjUxMDUwNTY3NjI2OTUsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo4MS41MDM4NjA0NzM2MzI4MSwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NTQzNDI2NTEzNjcxOSwiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjgyLjg5NDMxNzYyNjk1MzEyLCJsZWZ0Ijo3Ljg2NTU0NTI3MjgyNzE0OCwid2lkdGgiOjQwLjMyMTgyMzEyMDExNzE5LCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODQuMjkyMjg5NzMzODg2NzIsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6MTUuODcwMjk1NTI0NTk3MTY4LCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODUuNjgyNzQ2ODg3MjA3MDMsImxlZnQiOjkuNTQ5NjQxNjA5MTkxODk1LCJ3aWR0aCI6MzguOTg5NzA0MTMyMDgwMDgsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo4Ny4wNzMyMDQwNDA1MjczNCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC43MTg0OTQ0MTUyODMyLCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODguNDcxMTY4NTE4MDY2NCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42ODc0MTk4OTEzNTc0MiwiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjM0LjUxMzI3MTMzMTc4NzExLCJsZWZ0Ijo1MS45MzI3Nzc0MDQ3ODUxNTYsIndpZHRoIjo0MC42NDcwNTI3NjQ4OTI1OCwiaGVpZ2h0IjoyLjcxODY2NjU1MzQ5NzMxNDV9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjozOC43MjUxNTg2OTE0MDYyNSwibGVmdCI6NTEuOTMyNzc3NDA0Nzg1MTU2LCJ3aWR0aCI6NDAuNjQzMTM1MDcwODAwNzgsImhlaWdodCI6MS4yNTMyODg1MDc0NjE1NDc5fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6NDAuMTE1NjEyMDMwMDI5MywibGVmdCI6NTEuOTMyNzc3NDA0Nzg1MTU2LCJ3aWR0aCI6NDAuNjY0NTg1MTEzNTI1MzksImhlaWdodCI6MS4yNTMyODg1MDc0NjE1NDc5fX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNDMxIn19LHsib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjMwLCJzdWJ0eXBlIjp7fSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTWluZ3lpIiwiZmFtaWx5IjoiSmlhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik11aGFtbWFkIFVzbWFuIiwiZmFtaWx5IjoiRmFyaWQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSmVoYWQgQS4iLCJmYW1pbHkiOiJLaGFycmF6In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hbGxhcGFuZW5pIE1hbm9qIiwiZmFtaWx5IjoiS3VtYXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2hhdWhyYXQgUy4iLCJmYW1pbHkiOiJDaG9wcmEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW0iLCJmYW1pbHkiOiJKYW5nIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlkuTS4gSm9obiIsImZhbWlseSI6IkNoZXcifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2FtaXIgS3VtYXIiLCJmYW1pbHkiOiJLaGFuYWwifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiR3VhbmdoYW8iLCJmYW1pbHkiOiJDaGVuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFsaWNpYSBLeW91bmdqaW4iLCJmYW1pbHkiOiJBbiJ9fV0sImlkcyI6eyJpc3NuIjpbIjAwNDMtMTM1NCIsIjE4NzktMjQ0OCJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMTAxNi9qLndhdHJlcy4yMDIzLjEyMDYxMyIsInBtaWQiOiIzNzczODk0MCIsIm9wZW5hbGV4IjoiVzQzODY2MTg0NTkifSwidGl0bGUiOiJOYW5vYnViYmxlcyBpbiB3YXRlciBhbmQgd2FzdGV3YXRlciB0cmVhdG1lbnQgc3lzdGVtczogU21hbGwgYnViYmxlcyBtYWtpbmcgYmlnIGRpZmZlcmVuY2UiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAxNi9qLndhdHJlcy4yMDIzLjEyMDYxMyIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9wdXJlaG9zdC5iYXRoLmFjLnVrL3dzL2ZpbGVzLzI5NjM0MzkwMS9OYW5vYnViYmxlc19wcmVfcHJvb2YucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjMtMDktMTEiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA4IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJXYXRlciBSZXNlYXJjaCIsInZvbHVtZSI6MjQ1LCJpbXBhY3RGYWN0b3IiOiIxMy40NCJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjEyMDYxMywibGFzdFBhZ2UiOjEyMDYxM319LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJFbHNldmllciBCViJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MTExLCJjaXRlZEJ5Q291bnQiOjc1fX0sInF1b3RlIjoiU2V2ZXJhbCBzdHVkaWVzIGhhdmUgcHJvcG9zZWQgdGhhdCB0aGUgbmVnYXRpdmUgY2hhcmdlIG9mIE5CcyBhcmlzZXMgZHVlIHRvIHRoZWlyIHNtYWxsIHJhZGl1cyBhbmQgdGhlIGhpZ2ggcG9sYXJpdHkgb2YgaHlkcm94eWwgaW9ucywgd2hpY2ggbGVhZCB0byB0aGUgYWRzb3JwdGlvbiBvZiBkaXNzb2NpYXRlZCBoeWRyb3h5bCBpb25zIG9uIHRoZSBidWJibGUgc3VyZmFjZSwgd2hpbGUgaHlkcm9nZW4gaW9ucyBhcmUgZWFzaWx5IGh5ZHJhdGVkIGFuZCBwcmVmZXIgdG8gc3RheSBpbiB0aGUgYnVsayBsaXF1aWQuIEEgaGlnaGVyIGFic29sdXRlIHpldGEgcG90ZW50aWFsIGluZGljYXRlcyBoaWdoZXIgc3RhYmlsaXR5LCBhcyB0aGUgcmVwdWxzaXZlIGZvcmNlIGdlbmVyYXRlZCBieSBOQnPigJggc3VyZmFjZXMgcHJldmVudHMgdGhlbSBmcm9tIG1lcmdpbmcgd2l0aCBvbmUgYW5vdGhlciAoVXNoaWt1Ym8gZXQgYWwuLCAyMDEwKS4gMTEgU3VyZmFjZSBoeWRyb3Bob2JpY2l0eSBpcyBhbm90aGVyIGVzc2VudGlhbCBwcm9wZXJ0eSBvZiBOQnMgYXMgaXQgYWxsb3dzIE5CcyB0byBpbnRlcmFjdCB3aXRoIG90aGVyIHBhcnRpY2xlcyB0aHJvdWdoIGh5ZHJvcGhvYmljIGF0dHJhY3Rpb24uIEJOQnMgY2FuIGFkc29yYiBvbiBoeWRyb3Bob2JpYyBwYXJ0aWNsZXMgYW5kIGluY3JlYXNlIHRoZWlyIG1vYmlsaXR5IGluIHRoZSBsaXF1aWQsIHdoaWxlIGF0dGFjaGVkIE5CcyBjYW4gcHJvbW90ZSBwYXJ0aWNsZS1wYXJ0aWNsZSBvciBwYXJ0aWNsZS0gYnViYmxlIGludGVyYWN0aW9ucyB0aHJvdWdoIGNhcGlsbGFyeSBicmlkZ2luZyBlZmZlY3RzIHRoYXQgcHJvZHVjZSBhbiBhdHRyYWN0aXZlIGZvcmNlIGJldHdlZW4gdHdvIHN1cmZhY2VzIChIYW1wdG9uIGFuZCBOZ3V5ZW4sIDIwMTApLiAyLjEuMyBMb25nIHJlc2lkZW5jZSB0aW1lIGluIHdhdGVyOiBOQnPigJkgbG9uZy10ZXJtIHN0YWJpbGl0eSBBbW9uZyB0aGUgdmFyaW91cyBjaGFyYWN0ZXJpc3RpY3MgTkJzIHBvc3Nlc3MsIHRoZSBtb3N0IGFzdG9uaXNoaW5nIGlzIHRoZWlyIGxvbmdldml0eS4gVXBvbiBnZW5lcmF0aW9uLCBidWJibGVzIGdlbmVyYWxseSBtYWludGFpbiB0aGVpciBkaWFtZXRlcnMgZm9yIGEgY2VydGFpbiBhbW91bnQgb2YgdGltZSwgYWZ0ZXIgd2hpY2ggdGhlIGJ1YmJsZXMgYmVnaW4gdG8gZ3JvdyBhbmQgZXZlbnR1YWxseSBkaXNzb2x2ZSBpbiB0aGUgc29sdXRpb24uIEV4cGVyaW1lbnRhbCByZXN1bHRzIGhhdmUgY29uZmlybWVkIHRoYXQgTkJzIGFyZSBhYmxlIHRvIGV4aXN0IGluIHdhdGVyIGZyb20gZGF5cyB0byBtb250aHMuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxMSwiYmJveCI6eyJ0b3AiOjY4LjYzMTk2NTYzNzIwNzAzLCJsZWZ0Ijo5Ljc0MTE4MDQxOTkyMTg3NSwid2lkdGgiOjc3LjAxMDI1MzkwNjI1LCJoZWlnaHQiOjEuODQ5MjYzNjY4MDYwMzAyN319LHsicGFnZU51bWJlciI6MTEsImJib3giOnsidG9wIjo3MS44MjM4MjIwMjE0ODQzOCwibGVmdCI6OS43NDExODA0MTk5MjE4NzUsIndpZHRoIjo3Ny4wMDYzMDk1MDkyNzczNCwiaGVpZ2h0IjoxLjg0OTI2MzY2ODA2MDMwMjd9fSx7InBhZ2VOdW1iZXIiOjExLCJiYm94Ijp7InRvcCI6NzUuMDEyMjE0NjYwNjQ0NTMsImxlZnQiOjkuNzQxMTgwNDE5OTIxODc1LCJ3aWR0aCI6NzYuOTQ4Nzc2MjQ1MTE3MTksImhlaWdodCI6MS44NDkyNjM2NjgwNjAzMDI3fX0seyJwYWdlTnVtYmVyIjoxMSwiYmJveCI6eyJ0b3AiOjc4LjIwMDU5OTY3MDQxMDE2LCJsZWZ0Ijo5Ljc0MTE4MDQxOTkyMTg3NSwid2lkdGgiOjc3LjAwODI4NTUyMjQ2MDk0LCJoZWlnaHQiOjEuODQ5MjYzNjY4MDYwMzAyN319LHsicGFnZU51bWJlciI6MTEsImJib3giOnsidG9wIjo4MS4zODg5OTk5Mzg5NjQ4NCwibGVmdCI6OS43NDExODA0MTk5MjE4NzUsIndpZHRoIjo3Ny4wMTAyNDYyNzY4NTU0NywiaGVpZ2h0IjoxLjg0OTI2MzY2ODA2MDMwMjd9fSx7InBhZ2VOdW1iZXIiOjExLCJiYm94Ijp7InRvcCI6ODQuNTc2OTE5NTU1NjY0MDYsImxlZnQiOjkuNzQxMTgwNDE5OTIxODc1LCJ3aWR0aCI6MTkuNjE2NjI0ODMyMTUzMzIsImhlaWdodCI6MS44NDkyNjM2NjgwNjAzMDI3fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjg3LjQ4ODA1OTk5NzU1ODYsImxlZnQiOjQ3LjAxODI1MzMyNjQxNjAxNiwid2lkdGgiOjIuNDUyMTYxNzg4OTQwNDI5NywiaGVpZ2h0IjoxLjg0OTI2MzY2ODA2MDMwMjd9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MTEuNjQ4NzM4ODYxMDgzOTg0LCJsZWZ0Ijo5Ljc0MTE4MDQxOTkyMTg3NSwid2lkdGgiOjc2Ljk1MjcwNTM4MzMwMDc4LCJoZWlnaHQiOjEuODQ5MjY1NTc1NDA4OTM1NX19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoxNC44Mzk0MzQ2MjM3MTgyNjIsImxlZnQiOjkuNzQxMTgwNDE5OTIxODc1LCJ3aWR0aCI6NzYuOTQ4NzgzODc0NTExNzIsImhlaWdodCI6MS44NDkyNjU1NzU0MDg5MzU1fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjE4LjAyNzgzMzkzODU5ODYzMywibGVmdCI6OS43NDExODA0MTk5MjE4NzUsIndpZHRoIjo3Ni41MDYwNzI5OTgwNDY4OCwiaGVpZ2h0IjoxLjg0OTI2NTU3NTQwODkzNTV9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjEuMjE2MjE3MDQxMDE1NjI1LCJsZWZ0Ijo5Ljc0MTE4MDQxOTkyMTg3NSwid2lkdGgiOjc2Ljk4NTczMzAzMjIyNjU2LCJoZWlnaHQiOjEuODQ5MjY1NTc1NDA4OTM1NX19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyNC40MDQ2MDAxNDM0MzI2MTcsImxlZnQiOjkuNzQxMTgwNDE5OTIxODc1LCJ3aWR0aCI6MzUuMzc0MTk4OTEzNTc0MjIsImhlaWdodCI6MS44NDkyNjU1NzU0MDg5MzU1fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjMyLjIxMTMxMTM0MDMzMjAzLCJsZWZ0Ijo5Ljc0MTE4MDQxOTkyMTg3NSwid2lkdGgiOjUyLjQ2NDEyMjc3MjIxNjgsImhlaWdodCI6MS45MTk5NjUyNjcxODEzOTY1fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM2LjA1MjYyNzU2MzQ3NjU2LCJsZWZ0Ijo5Ljc0MTE4MDQxOTkyMTg3NSwid2lkdGgiOjc3LjAwODI4NTUyMjQ2MDk0LCJoZWlnaHQiOjEuODQ5MjY3NDgyNzU3NTY4NH19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozOS4yNDEwMTYzODc5Mzk0NSwibGVmdCI6OS43NDExODA0MTk5MjE4NzUsIndpZHRoIjo3Ny4wMDgyODU1MjI0NjA5NCwiaGVpZ2h0IjoxLjg0OTI2NzQ4Mjc1NzU2ODR9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDIuNDI5NDA5MDI3MDk5NjEsImxlZnQiOjkuNzQxMTgwNDE5OTIxODc1LCJ3aWR0aCI6NzcuMDAyNDAzMjU5Mjc3MzQsImhlaWdodCI6MS44NDkyNjc0ODI3NTc1Njg0fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQ1LjYxNzc5NDAzNjg2NTIzNCwibGVmdCI6OS43NDExODA0MTk5MjE4NzUsIndpZHRoIjo3Ny4wMTAyNDYyNzY4NTU0NywiaGVpZ2h0IjoxLjg0OTI2NzQ4Mjc1NzU2ODR9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIxMjA2MjQifX1d. This charge arises predominantly from the preferential adsorption of hydroxide ions (OH⁻) at the gas-water interface, forming a robust electrical double layer that overwhelms attractive van der Waals forces. Studies confirm that the magnitude of zeta potential increases with solution pH, enhancing stability in neutral to alkaline conditions typical of aquaculture systems. Lower OH⁻ availability in acidic media reduces repulsion and promotes aggregation (Wu et al., 2021)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjMwLCJzdWJ0eXBlIjp7fSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSmlhamlhIiwiZmFtaWx5IjoiV3UifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiS2VqaWEiLCJmYW1pbHkiOiJaaGFuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJDaGVuZyIsImZhbWlseSI6IkNlbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJYaWFvZ2FuZyIsImZhbWlseSI6Ild1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlJ1eWluIiwiZmFtaWx5IjoiTWFvIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Illpbmd5aW5nIiwiZmFtaWx5IjoiWmhlbmcifX1dLCJpZHMiOnsiaXNzbiI6WyIyMTkxLTA4NTUiXSwiaXNibiI6W10sImRvaSI6IjEwLjExODYvczEzNTY4LTAyMS0wMTI1NC0wIiwibWFnIjoiMzE3NTY4MzU2MSIsInBtaWQiOiIzNDE4NDEzNyIsInBtY2lkIjoiODIzOTEwOSIsIm9wZW5hbGV4IjoiVzMxNzU2ODM1NjEifSwidGl0bGUiOiJSb2xlIG9mIGJ1bGsgbmFub2J1YmJsZXMgaW4gcmVtb3Zpbmcgb3JnYW5pYyBwb2xsdXRhbnRzIGluIHdhc3Rld2F0ZXIgdHJlYXRtZW50IiwiYWJzdHJhY3QiOiJBYnN0cmFjdCBUaGUgb2NjdXJyZW5jZSBvZiBhIHZhcmlldHkgb2Ygb3JnYW5pYyBwb2xsdXRhbnRzIGhhcyBjb21wbGljYXRlZCB3YXN0ZXdhdGVyIHRyZWF0bWVudDsgdGh1cywgdGhlIHNlYXJjaCBmb3Igc3VzdGFpbmFibGUgYW5kIGVmZmVjdGl2ZSB0cmVhdG1lbnQgdGVjaG5vbG9neSBoYXMgZHJhd24gc2lnbmlmaWNhbnQgYXR0ZW50aW9uLiBJbiByZWNlbnQgeWVhcnMsIGJ1bGsgbmFub2J1YmJsZXMsIHdoaWNoIGhhdmUgZXh0cmFvcmRpbmFyeSBwcm9wZXJ0aWVzIGRpZmZlcmluZyBmcm9tIHRob3NlIG9mIG1pY3JvYnViYmxlcywgaW5jbHVkaW5nIGhpZ2ggc3RhYmlsaXR5IGFuZCBsb25nIHJlc2lkZW5jZSB0aW1lcyBpbiB3YXRlciwgbGFyZ2Ugc3BlY2lmaWMgc3VyZmFjZSBhcmVhcywgaGlnaCBnYXMgdHJhbnNmZXIgZWZmaWNpZW5jeSBhbmQgaW50ZXJmYWNlIHBvdGVudGlhbCwgYW5kIHRoZSBjYXBhYmlsaXR5IHRvIGdlbmVyYXRlIGZyZWUgcmFkaWNhbHMsIGhhdmUgc2hvd24gYXR0cmFjdGl2ZSB0ZWNobm9sb2dpY2FsIGFkdmFudGFnZXMgYW5kIHByb21pc2luZyBhcHBsaWNhdGlvbiBwcm9zcGVjdHMgZm9yIHdhc3Rld2F0ZXIgdHJlYXRtZW50LiBJbiB0aGlzIHJldmlldywgdGhlIGJhc2ljIGNoYXJhY3RlcmlzdGljcyBvZiBidWxrIG5hbm9idWJibGVzIGFyZSBzdW1tYXJpemVkIGluIGRldGFpbCwgYW5kIHJlY2VudCBmaW5kaW5ncyByZWxhdGVkIHRvIHRoZWlyIGltcGxlbWVudGF0aW9uIHBhdGh3YXlzIGFuZCBtZWNoYW5pc21zIGluIG9yZ2FuaWMgd2FzdGV3YXRlciB0cmVhdG1lbnQgYXJlIHN5c3RlbWF0aWNhbGx5IGRpc2N1c3NlZCwgd2hpY2ggaW5jbHVkZXMgaW1wcm92aW5nIHRoZSBhaXIgZmxvdGF0aW9uIHByb2Nlc3MsIGluY3JlYXNpbmcgd2F0ZXIgYWVyYXRpb24gdG8gcHJvbW90ZSBhZXJvYmljIGJpb2xvZ2ljYWwgdGVjaG5vbG9naWVzIGluY2x1ZGluZyBiaW9sb2dpY2FsIGFjdGl2YXRlZCBjYXJib24sIGFjdGl2YXRlZCBzbHVkZ2UsIGFuZCBtZW1icmFuZSBiaW9yZWFjdG9ycywgYW5kIGdlbmVyYXRpbmcgYWN0aXZlIGZyZWUgcmFkaWNhbHMgdGhhdCBveGlkaXNlIG9yZ2FuaWMgY29tcG91bmRzLiBGaW5hbGx5LCB0aGUgY3VycmVudCB0ZWNobm9sb2dpY2FsIGRpZmZpY3VsdGllcyBvZiBidWxrIG5hbm9idWJibGVzIGFyZSBhbmFseXNlZCwgYW5kIGZ1dHVyZSBmb2N1cyBhcmVhcyBmb3IgcmVzZWFyY2ggb24gYnVsayBuYW5vYnViYmxlIHRlY2hub2xvZ3kgYXJlIGFsc28gcHJvcG9zZWQuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjExODYvczEzNTY4LTAyMS0wMTI1NC0wIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2FtYi1leHByZXNzLnNwcmluZ2Vyb3Blbi5jb20vY291bnRlci9wZGYvMTAuMTE4Ni9zMTM1NjgtMDIxLTAxMjU0LTAiLCJpc3N1ZWREYXRlIjoiMjAyMS0wNi0yOCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkFNQiBFeHByZXNzIiwiaXNzdWUiOjEsInZvbHVtZSI6MTEsImltcGFjdEZhY3RvciI6IjQuMjYifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiU3ByaW5nZXIgTmF0dXJlIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo3NiwiY2l0ZWRCeUNvdW50Ijo3NX19LCJxdW90ZSI6IlRoZSBtb2RlbCBzdWNjZXNzZnVsbHkgZWx1Y2lkYXRlcyByZXB1bHNpdmUgZm9yY2VzICBiZXR3ZWVuIE9I4oiSIG9uIHRoZSBzdXJmYWNlIGFyZSBzdWZmaWNpZW50IHRvIHVsdGktbWF0ZWx5IGJhbGFuY2UgdGhlIHN1cmZhY2UgdGVuc2lvbiBmb3JjZXMgYXQgbmFub2J1YmJsZSAgc2l6ZXMsIHRodXMsIG1ha2luZyBuYW5vYnViYmxlcyBleGhpYml0IGdyZWF0IHN0YWJpbC1pdHkuIEFzIHNob3duIGluIHByZXZpb3VzIGxpdGVyYXR1cmVzLCB6ZXRhIHBvdGVudGlhbCB2YWwtdWVzIGNoYW5nZSB3aXRoIHRoZSBwSCBvZiB0aGUgbWVkaXVtIHZhcnksIGV4aGliaXRpbmcgIGEgdGVuZGVuY3kgdGhhdCB0aGUgbmVnYXRpdml0eSBvZiB6ZXRhIHBvdGVudGlhbCBpbmNyZWFzZXMgIHdpdGggc29sdXRpb24gcEggaW5jcmVhc2Ugd2hpY2ggcmVzdWx0cyBpbiBncmVhdGVyIGJ1Yi1ibGUgc3VyZmFjZSBjaGFyZ2UgYW5kIG1ha2UgYnViYmxlcyBtb3JlIHN0YWJsZSBkdWUgdG8gIGVsZWN0cm9zdGF0aWMgcmVwdWxzaW9uIChNZWVnb2RhIGV0IGFsLiAyMDE4KS4gTmFub2J1Yi1ibGVzIGdlbmVyYWxseSBzaG93IGhpZ2ggbmVnYXRpdmUgemV0YSBwb3RlbnRpYWwgb3dpbmcgdG8gIHRoZSBleGNlc3Mgb2YgaHlkcm94aWRlIGlvbnMgKE9I4oiSKSByZWxhdGl2ZSB0byBoeWRyb2dlbiAgaW9ucyAoSCspIGF0IHRoZSBnYXPigJN3YXRlciBpbnRlcmZhY2Ugb2YgYnViYmxlcyB1bmRlciAgd2lkZSByYW5nZSBvZiBzb2x1dGlvbiBwSCwgYnV0IG5hbm9idWJibGVzIGZvcm1lZCBpbiAgYWNpZGljIGVudmlyb25tZW50cyBhbHdheXMgc2hvdyBsb3dlciBzdGFiaWxpdHkgY29tLXBhcmVkIHdpdGggaW4gbmV1dHJhbCBvciBhbGthbGluZSBjb25kaXRpb25zIGJlY2F1c2UgYSAgbGFjayBvZiBPSOKIkiBhcmUgcmVxdWlyZWQgdG8gc3RhYmlsaXplIHRoZSBlbGVjdHJpYyBkb3VibGUgIGxheWVyIChOaXJtYWxrYXIgZXQgYWwuIDIwMTgpLCB0aGUgemV0YSBwb3RlbnRpYWwgb2YgbmFuby1idWJibGVzIGdlbmVyYXRlZCBpbiBzdHJvbmcgYWNpZGljIG1lZGl1bSBiZWxvdyBwSCAzICB0ZW5kcyB0byBiZSBwb3NpdGl2ZSBhbmQgbG93IHdoaWNoIGNhdXNlcyBoaWdoZXIgcG9zc2ktYmlsaXR5IGZvciBidWJibGUgY29hbGVzY2VuY2UgKFpoYW5nIGV0IGFsLiAyMDIwKS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjoxMy45NjgzMDU1ODc3Njg1NTUsImxlZnQiOjUyLjUzNDM0NzUzNDE3OTY5LCJ3aWR0aCI6MzguMzI2OTE1NzQwOTY2OCwiaGVpZ2h0IjoxLjgzNzY1NjYxNzE2NDYxMTh9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjoxNS40NzczMzQwMjI1MjE5NzMsImxlZnQiOjUxLjE5MDk3MTM3NDUxMTcyLCJ3aWR0aCI6MzkuMjg1MTYzODc5Mzk0NTMsImhlaWdodCI6MS44NDY1NjE1NTEwOTQwNTUyfX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MTcuMDAzNTYyOTI3MjQ2MDk0LCJsZWZ0Ijo1MS4xOTA0MzczMTY4OTQ1Mywid2lkdGgiOjM5LjY2OTEwNTUyOTc4NTE1NiwiaGVpZ2h0IjoxLjgzNzY1NjYxNzE2NDYxMTh9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjoxOC41MjAyNzcwMjMzMTU0MywibGVmdCI6NTEuMTkwNDM3MzE2ODk0NTMsIndpZHRoIjozOS4yODU2OTQxMjIzMTQ0NSwiaGVpZ2h0IjoxLjgzODI2NjI1MzQ3MTM3NDV9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjoyMC4wMzgyMTE4MjI1MDk3NjYsImxlZnQiOjUxLjE5MDQzNzMxNjg5NDUzLCJ3aWR0aCI6MzkuMjg1Njk0MTIyMzE0NDUsImhlaWdodCI6MS44Mzc2NTY2MTcxNjQ2MTE4fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MjEuNTU1NTM2MjcwMTQxNiwibGVmdCI6NTEuMTkwNDM3MzE2ODk0NTMsIndpZHRoIjozOS42NzUyMTY2NzQ4MDQ2OSwiaGVpZ2h0IjoxLjgzNzY1NjYxNzE2NDYxMTh9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjoyMy4wNzIyNDg0NTg4NjIzMDUsImxlZnQiOjUxLjE5MDQzNzMxNjg5NDUzLCJ3aWR0aCI6MzkuNjcwMzI2MjMyOTEwMTU2LCJoZWlnaHQiOjEuODM3NjU2NjE3MTY0NjExOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjI0LjU4ODk2MjU1NDkzMTY0LCJsZWZ0Ijo1MS4xOTA0MzczMTY4OTQ1Mywid2lkdGgiOjM5LjI4NTY5NDEyMjMxNDQ1LCJoZWlnaHQiOjEuODM4ODc1ODg5Nzc4MTM3Mn19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjI2LjEwNzUwNzcwNTY4ODQ3NywibGVmdCI6NTEuMTkwNDM3MzE2ODk0NTMsIndpZHRoIjozOS42NzIyNTI2NTUwMjkzLCJoZWlnaHQiOjEuODM3NjU2NjE3MTY0NjExOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjI3LjYyNDIyMTgwMTc1NzgxMiwibGVmdCI6NTEuMTkwNDM3MzE2ODk0NTMsIndpZHRoIjozOS4yODU2OTQxMjIzMTQ0NSwiaGVpZ2h0IjoxLjgzODI2NjI1MzQ3MTM3NDV9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjoyOS4xNDIxNTY2MDA5NTIxNSwibGVmdCI6NTEuMTkwNDM3MzE2ODk0NTMsIndpZHRoIjozOS42NzUyMzU3NDgyOTEwMTYsImhlaWdodCI6MS44Mzc2NTY2MTcxNjQ2MTE4fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MzAuNjUwNTczNzMwNDY4NzUsImxlZnQiOjUxLjE5MDQzNzMxNjg5NDUzLCJ3aWR0aCI6MzkuNjcxNzI2MjI2ODA2NjQsImhlaWdodCI6MS44NDY1NTg4MDkyODAzOTU1fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MzIuMTY3OTAwMDg1NDQ5MjIsImxlZnQiOjUxLjE4OTg5OTQ0NDU4MDA4LCJ3aWR0aCI6MzkuNjY5OTQwOTQ4NDg2MzMsImhlaWdodCI6MS44NDY1NTg4MDkyODAzOTU1fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MzMuNjkzNTE1Nzc3NTg3ODksImxlZnQiOjUxLjE5MDg2MDc0ODI5MTAxNiwid2lkdGgiOjM5LjY3NTcyMDIxNDg0Mzc1LCJoZWlnaHQiOjEuODM3NjU0NTkwNjA2Njg5NX19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjM1LjIxMDIzNTU5NTcwMzEyNSwibGVmdCI6NTEuMTkwODYwNzQ4MjkxMDE2LCJ3aWR0aCI6MzkuMjg1MjcwNjkwOTE3OTcsImhlaWdodCI6MS44Mzg4NzQxMDE2Mzg3OTR9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjozNi43Mjg3Nzg4MzkxMTEzMywibGVmdCI6NTEuMTkwNDM3MzE2ODk0NTMsIndpZHRoIjozOS42Njg5NzIwMTUzODA4NiwiaGVpZ2h0IjoxLjgzNzY1NDU5MDYwNjY4OTV9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjozOC4yMzcxNzg4MDI0OTAyMzQsImxlZnQiOjUxLjE5MDQzNzMxNjg5NDUzLCJ3aWR0aCI6MzkuNjY4ODA3OTgzMzk4NDQsImhlaWdodCI6MS44NDY1ODAwMjg1MzM5MzU1fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MzkuNzYyODIxMTk3NTA5NzY2LCJsZWZ0Ijo1MS4xODk5NTI4NTAzNDE4LCJ3aWR0aCI6MzkuMjg2MTgyNDAzNTY0NDUsImhlaWdodCI6MS44MzgyNjA1MzE0MjU0NzZ9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo0MS4yODA3NTAyNzQ2NTgyLCJsZWZ0Ijo1MS4xOTA0MzczMTY4OTQ1Mywid2lkdGgiOjM5LjY3MjczMzMwNjg4NDc2NiwiaGVpZ2h0IjoxLjgzNzY1NDU5MDYwNjY4OTV9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo0Mi43OTc0NzAwOTI3NzM0NCwibGVmdCI6NTEuMTkwNDM3MzE2ODk0NTMsIndpZHRoIjozOS4yODU2OTQxMjIzMTQ0NSwiaGVpZ2h0IjoxLjgzODI2MDUzMTQyNTQ3Nn19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjQ0LjMxNTM5OTE2OTkyMTg3NSwibGVmdCI6NTEuMTkwNDM3MzE2ODk0NTMsIndpZHRoIjozOS4yODU2OTQxMjIzMTQ0NSwiaGVpZ2h0IjoxLjgzNzY1NDU5MDYwNjY4OTV9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIzIn19XQ%3D%3D. Furthermore, surface hydrophobicity allows nanobubbles to interact with organic particles via hydrophobic attraction and capillary bridging, aiding in pollutant mobilization without premature dissolution (Jia et al., 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjMwLCJzdWJ0eXBlIjp7fSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTWluZ3lpIiwiZmFtaWx5IjoiSmlhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik11aGFtbWFkIFVzbWFuIiwiZmFtaWx5IjoiRmFyaWQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSmVoYWQgQS4iLCJmYW1pbHkiOiJLaGFycmF6In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hbGxhcGFuZW5pIE1hbm9qIiwiZmFtaWx5IjoiS3VtYXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2hhdWhyYXQgUy4iLCJmYW1pbHkiOiJDaG9wcmEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW0iLCJmYW1pbHkiOiJKYW5nIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlkuTS4gSm9obiIsImZhbWlseSI6IkNoZXcifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2FtaXIgS3VtYXIiLCJmYW1pbHkiOiJLaGFuYWwifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiR3VhbmdoYW8iLCJmYW1pbHkiOiJDaGVuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFsaWNpYSBLeW91bmdqaW4iLCJmYW1pbHkiOiJBbiJ9fV0sImlkcyI6eyJpc3NuIjpbIjAwNDMtMTM1NCIsIjE4NzktMjQ0OCJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMTAxNi9qLndhdHJlcy4yMDIzLjEyMDYxMyIsInBtaWQiOiIzNzczODk0MCIsIm9wZW5hbGV4IjoiVzQzODY2MTg0NTkifSwidGl0bGUiOiJOYW5vYnViYmxlcyBpbiB3YXRlciBhbmQgd2FzdGV3YXRlciB0cmVhdG1lbnQgc3lzdGVtczogU21hbGwgYnViYmxlcyBtYWtpbmcgYmlnIGRpZmZlcmVuY2UiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAxNi9qLndhdHJlcy4yMDIzLjEyMDYxMyIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9wdXJlaG9zdC5iYXRoLmFjLnVrL3dzL2ZpbGVzLzI5NjM0MzkwMS9OYW5vYnViYmxlc19wcmVfcHJvb2YucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjMtMDktMTEiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA4IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJXYXRlciBSZXNlYXJjaCIsInZvbHVtZSI6MjQ1LCJpbXBhY3RGYWN0b3IiOiIxMy40NCJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjEyMDYxMywibGFzdFBhZ2UiOjEyMDYxM319LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJFbHNldmllciBCViJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MTExLCJjaXRlZEJ5Q291bnQiOjc1fX0sInF1b3RlIjoiU2V2ZXJhbCBzdHVkaWVzIGhhdmUgcHJvcG9zZWQgdGhhdCB0aGUgbmVnYXRpdmUgY2hhcmdlIG9mIE5CcyBhcmlzZXMgZHVlIHRvIHRoZWlyIHNtYWxsIHJhZGl1cyBhbmQgdGhlIGhpZ2ggcG9sYXJpdHkgb2YgaHlkcm94eWwgaW9ucywgd2hpY2ggbGVhZCB0byB0aGUgYWRzb3JwdGlvbiBvZiBkaXNzb2NpYXRlZCBoeWRyb3h5bCBpb25zIG9uIHRoZSBidWJibGUgc3VyZmFjZSwgd2hpbGUgaHlkcm9nZW4gaW9ucyBhcmUgZWFzaWx5IGh5ZHJhdGVkIGFuZCBwcmVmZXIgdG8gc3RheSBpbiB0aGUgYnVsayBsaXF1aWQuIEEgaGlnaGVyIGFic29sdXRlIHpldGEgcG90ZW50aWFsIGluZGljYXRlcyBoaWdoZXIgc3RhYmlsaXR5LCBhcyB0aGUgcmVwdWxzaXZlIGZvcmNlIGdlbmVyYXRlZCBieSBOQnPigJggc3VyZmFjZXMgcHJldmVudHMgdGhlbSBmcm9tIG1lcmdpbmcgd2l0aCBvbmUgYW5vdGhlciAoVXNoaWt1Ym8gZXQgYWwuLCAyMDEwKS4gMTEgU3VyZmFjZSBoeWRyb3Bob2JpY2l0eSBpcyBhbm90aGVyIGVzc2VudGlhbCBwcm9wZXJ0eSBvZiBOQnMgYXMgaXQgYWxsb3dzIE5CcyB0byBpbnRlcmFjdCB3aXRoIG90aGVyIHBhcnRpY2xlcyB0aHJvdWdoIGh5ZHJvcGhvYmljIGF0dHJhY3Rpb24uIEJOQnMgY2FuIGFkc29yYiBvbiBoeWRyb3Bob2JpYyBwYXJ0aWNsZXMgYW5kIGluY3JlYXNlIHRoZWlyIG1vYmlsaXR5IGluIHRoZSBsaXF1aWQsIHdoaWxlIGF0dGFjaGVkIE5CcyBjYW4gcHJvbW90ZSBwYXJ0aWNsZS1wYXJ0aWNsZSBvciBwYXJ0aWNsZS0gYnViYmxlIGludGVyYWN0aW9ucyB0aHJvdWdoIGNhcGlsbGFyeSBicmlkZ2luZyBlZmZlY3RzIHRoYXQgcHJvZHVjZSBhbiBhdHRyYWN0aXZlIGZvcmNlIGJldHdlZW4gdHdvIHN1cmZhY2VzIChIYW1wdG9uIGFuZCBOZ3V5ZW4sIDIwMTApLiAyLjEuMyBMb25nIHJlc2lkZW5jZSB0aW1lIGluIHdhdGVyOiBOQnPigJkgbG9uZy10ZXJtIHN0YWJpbGl0eSBBbW9uZyB0aGUgdmFyaW91cyBjaGFyYWN0ZXJpc3RpY3MgTkJzIHBvc3Nlc3MsIHRoZSBtb3N0IGFzdG9uaXNoaW5nIGlzIHRoZWlyIGxvbmdldml0eS4gVXBvbiBnZW5lcmF0aW9uLCBidWJibGVzIGdlbmVyYWxseSBtYWludGFpbiB0aGVpciBkaWFtZXRlcnMgZm9yIGEgY2VydGFpbiBhbW91bnQgb2YgdGltZSwgYWZ0ZXIgd2hpY2ggdGhlIGJ1YmJsZXMgYmVnaW4gdG8gZ3JvdyBhbmQgZXZlbnR1YWxseSBkaXNzb2x2ZSBpbiB0aGUgc29sdXRpb24uIEV4cGVyaW1lbnRhbCByZXN1bHRzIGhhdmUgY29uZmlybWVkIHRoYXQgTkJzIGFyZSBhYmxlIHRvIGV4aXN0IGluIHdhdGVyIGZyb20gZGF5cyB0byBtb250aHMuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxMSwiYmJveCI6eyJ0b3AiOjY4LjYzMTk2NTYzNzIwNzAzLCJsZWZ0Ijo5Ljc0MTE4MDQxOTkyMTg3NSwid2lkdGgiOjc3LjAxMDI1MzkwNjI1LCJoZWlnaHQiOjEuODQ5MjYzNjY4MDYwMzAyN319LHsicGFnZU51bWJlciI6MTEsImJib3giOnsidG9wIjo3MS44MjM4MjIwMjE0ODQzOCwibGVmdCI6OS43NDExODA0MTk5MjE4NzUsIndpZHRoIjo3Ny4wMDYzMDk1MDkyNzczNCwiaGVpZ2h0IjoxLjg0OTI2MzY2ODA2MDMwMjd9fSx7InBhZ2VOdW1iZXIiOjExLCJiYm94Ijp7InRvcCI6NzUuMDEyMjE0NjYwNjQ0NTMsImxlZnQiOjkuNzQxMTgwNDE5OTIxODc1LCJ3aWR0aCI6NzYuOTQ4Nzc2MjQ1MTE3MTksImhlaWdodCI6MS44NDkyNjM2NjgwNjAzMDI3fX0seyJwYWdlTnVtYmVyIjoxMSwiYmJveCI6eyJ0b3AiOjc4LjIwMDU5OTY3MDQxMDE2LCJsZWZ0Ijo5Ljc0MTE4MDQxOTkyMTg3NSwid2lkdGgiOjc3LjAwODI4NTUyMjQ2MDk0LCJoZWlnaHQiOjEuODQ5MjYzNjY4MDYwMzAyN319LHsicGFnZU51bWJlciI6MTEsImJib3giOnsidG9wIjo4MS4zODg5OTk5Mzg5NjQ4NCwibGVmdCI6OS43NDExODA0MTk5MjE4NzUsIndpZHRoIjo3Ny4wMTAyNDYyNzY4NTU0NywiaGVpZ2h0IjoxLjg0OTI2MzY2ODA2MDMwMjd9fSx7InBhZ2VOdW1iZXIiOjExLCJiYm94Ijp7InRvcCI6ODQuNTc2OTE5NTU1NjY0MDYsImxlZnQiOjkuNzQxMTgwNDE5OTIxODc1LCJ3aWR0aCI6MTkuNjE2NjI0ODMyMTUzMzIsImhlaWdodCI6MS44NDkyNjM2NjgwNjAzMDI3fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjg3LjQ4ODA1OTk5NzU1ODYsImxlZnQiOjQ3LjAxODI1MzMyNjQxNjAxNiwid2lkdGgiOjIuNDUyMTYxNzg4OTQwNDI5NywiaGVpZ2h0IjoxLjg0OTI2MzY2ODA2MDMwMjd9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MTEuNjQ4NzM4ODYxMDgzOTg0LCJsZWZ0Ijo5Ljc0MTE4MDQxOTkyMTg3NSwid2lkdGgiOjc2Ljk1MjcwNTM4MzMwMDc4LCJoZWlnaHQiOjEuODQ5MjY1NTc1NDA4OTM1NX19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoxNC44Mzk0MzQ2MjM3MTgyNjIsImxlZnQiOjkuNzQxMTgwNDE5OTIxODc1LCJ3aWR0aCI6NzYuOTQ4NzgzODc0NTExNzIsImhlaWdodCI6MS44NDkyNjU1NzU0MDg5MzU1fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjE4LjAyNzgzMzkzODU5ODYzMywibGVmdCI6OS43NDExODA0MTk5MjE4NzUsIndpZHRoIjo3Ni41MDYwNzI5OTgwNDY4OCwiaGVpZ2h0IjoxLjg0OTI2NTU3NTQwODkzNTV9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjEuMjE2MjE3MDQxMDE1NjI1LCJsZWZ0Ijo5Ljc0MTE4MDQxOTkyMTg3NSwid2lkdGgiOjc2Ljk4NTczMzAzMjIyNjU2LCJoZWlnaHQiOjEuODQ5MjY1NTc1NDA4OTM1NX19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyNC40MDQ2MDAxNDM0MzI2MTcsImxlZnQiOjkuNzQxMTgwNDE5OTIxODc1LCJ3aWR0aCI6MzUuMzc0MTk4OTEzNTc0MjIsImhlaWdodCI6MS44NDkyNjU1NzU0MDg5MzU1fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjMyLjIxMTMxMTM0MDMzMjAzLCJsZWZ0Ijo5Ljc0MTE4MDQxOTkyMTg3NSwid2lkdGgiOjUyLjQ2NDEyMjc3MjIxNjgsImhlaWdodCI6MS45MTk5NjUyNjcxODEzOTY1fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM2LjA1MjYyNzU2MzQ3NjU2LCJsZWZ0Ijo5Ljc0MTE4MDQxOTkyMTg3NSwid2lkdGgiOjc3LjAwODI4NTUyMjQ2MDk0LCJoZWlnaHQiOjEuODQ5MjY3NDgyNzU3NTY4NH19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozOS4yNDEwMTYzODc5Mzk0NSwibGVmdCI6OS43NDExODA0MTk5MjE4NzUsIndpZHRoIjo3Ny4wMDgyODU1MjI0NjA5NCwiaGVpZ2h0IjoxLjg0OTI2NzQ4Mjc1NzU2ODR9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDIuNDI5NDA5MDI3MDk5NjEsImxlZnQiOjkuNzQxMTgwNDE5OTIxODc1LCJ3aWR0aCI6NzcuMDAyNDAzMjU5Mjc3MzQsImhlaWdodCI6MS44NDkyNjc0ODI3NTc1Njg0fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQ1LjYxNzc5NDAzNjg2NTIzNCwibGVmdCI6OS43NDExODA0MTk5MjE4NzUsIndpZHRoIjo3Ny4wMTAyNDYyNzY4NTU0NywiaGVpZ2h0IjoxLjg0OTI2NzQ4Mjc1NzU2ODR9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIxMjA2MjQifX1d. This charged interface also fosters charge separation, contributing to an energy barrier that establishes a local minimum in the free-energy landscape for bubble existence (Sakr et al., 2025)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1hcndhIiwiZmFtaWx5IjoiU2FrciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNb2hhbWVkIE0uIiwiZmFtaWx5IjoiTW9oYW1lZCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNb2hhbWVkIElicmFoaW0iLCJmYW1pbHkiOiJLaXpoaXNzZXJpIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik11bmplZCBBLiIsImZhbWlseSI6Ik1hcmFxYSJ9fV0sImlkcyI6eyJpc3NuIjpbIjIwNDUtMjMyMiJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMTAzOC9zNDE1OTgtMDI1LTIxMjkzLXkiLCJwbWlkIjoiNDExMzY2MTQiLCJvcGVuYWxleCI6Ilc0NDE1NTAyNTY4In0sInRpdGxlIjoiQ2hhcmFjdGVyaXphdGlvbiBvZiBhaXIgc3VibWljcm9idWJibGVzIGZvciB3YXRlciB0cmVhdG1lbnQgdW5kZXIgZGlmZmVyZW50IGdlbmVyYXRpb24gY29uZGl0aW9ucyIsImFic3RyYWN0IjoiTWljcm8tbmFub2J1YmJsZSAoTU5CKSB0ZWNobm9sb2d5IGhhcyBiZWVuIHByb3ZlbiB0byBiZSBlZmZlY3RpdmUgaW4gd2F0ZXIgYW5kIHdhc3Rld2F0ZXIgdHJlYXRtZW50LiBTdWJtaWNyb2J1YmJsZXMgKFNNQnMpIGFyZSBjb25zaWRlcmVkIHRvIGJlIGEgc3ViZ3JvdXAgb2YgTU5CcyByYW5naW5nIGZyb20gMSB0byAxMCDOvG0gd2l0aCBsaW1pdGVkIHN0dWRpZXMgcmVsYXRlZCB0byB0aGVpciBmdW5kYW1lbnRhbCBwcm9wZXJ0aWVzLiBUaGlzIHN0dWR5IGZvY3VzZWQgb24gdGhlIGZ1bmRhbWVudGFsIGNoYXJhY3RlcmlzdGljcyBvZiBTTUJzIGFuZCB0aGUgZWZmZWN0IG9mIGdlbmVyYXRpb24gY29uZGl0aW9ucyBzdWNoIGFzIHRlbXBlcmF0dXJlLCBhZXJhdGlvbiB0aW1lLCBhbmQgd2F0ZXIgdm9sdW1lIG9uIHRoZWlyIHByb3BlcnRpZXMuIFNNQnMgd2VyZSBnZW5lcmF0ZWQgdW5kZXIgaGlnaCBwcmVzc3VyZSB1c2luZyBzaGVhciBmb3JjZSBhbmQgdGhlaXIgc2l6ZSBhbmQgZGlzdHJpYnV0aW9uIHdlcmUgbWVhc3VyZWQgdXNpbmcgdGhlIGR5bmFtaWMgbGlnaHQgc2NhdHRlcmluZyBtZXRob2QuIERpc3NvbHZlZCBveHlnZW4gKERPKSBjb25jZW50cmF0aW9ucyB3ZXJlIG1vbml0b3JlZCBkdXJpbmcgYW5kIGFmdGVyIHRoZSBnZW5lcmF0aW9uLiBUaGUgemV0YSBwb3RlbnRpYWwgb2YgdGhlIGdlbmVyYXRlZCBidWJibGVzIHdhcyBhbHNvIG1lYXN1cmVkIHRvIGFzc2VzcyBidWJibGVzIHN0YWJpbGl0eS4gU01CcyB3aXRoIGEgbWVkaWFuIGRpYW1ldGVyIG9mIDIgzrxtIHBlcnNpc3RlZCBpbiB3YXRlciBldmVuIGFmdGVyIHRoZSBnZW5lcmF0b3Igd2FzIHN0b3BwZWQsIGluZGljYXRpbmcgdGhlIGhpZ2ggbG9uZ2V2aXR5IG9mIFNNQnMgaW4gd2F0ZXIuIFJlZ2FyZGxlc3Mgb2YgdGhlIGFlcmF0aW9uIHRpbWUgb3Igd2F0ZXIgdm9sdW1lIHVzZWQsIHRoZSB6ZXRhIHBvdGVudGlhbCBvZiBTTUJzIHdhcyBoaWdobHkgbmVnYXRpdmUgd2l0aCBhdmVyYWdlIHZhbHVlcyByYW5nZWQgYmV0d2VlbiDiiJIgMjggYW5kIOKIkiAzMCBtViwgaW5kaWNhdGluZyBoaWdoIHN0YWJpbGl0eSBpbiB3YXRlci4gRE8gY29uY2VudHJhdGlvbiBpbmNyZWFzZWQgYnkgdXAgdG8gMS41IGZvbGRzIHdpdGhpbiBhIGZldyBtaW51dGVzIG9mIGdlbmVyYXRpb24gYW5kIHNsaWdodGx5IGRlY3JlYXNlZCBvdmVyIDEgaC4gUmVzdWx0cyBkZW1vbnN0cmF0ZSB0aGF0IGFpciBTTUJzIGFyZSBzdGFibGUgd2l0aCBsb25nIGxpZmVzcGFuIGFuZCBoaWdoIERPIGNvbmNlbnRyYXRpb24uIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjEwMzgvczQxNTk4LTAyNS0yMTI5My15IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5uYXR1cmUuY29tL2FydGljbGVzL3M0MTU5OC0wMjUtMjEyOTMteS5wZGYiLCJpc3N1ZWREYXRlIjoiMjAyNS0xMC0yNCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IlNjaWVudGlmaWMgUmVwb3J0cyIsImlzc3VlIjoxLCJ2b2x1bWUiOjE1LCJpbXBhY3RGYWN0b3IiOiI0LjkxIn0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6Ik5hdHVyZSBQb3J0Zm9saW8ifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjY0LCJjaXRlZEJ5Q291bnQiOjB9fSwicXVvdGUiOiJUaGUgdmFsdWVzIGNvdWxkIGRpZmZlciBmcm9tIG9uZSBleHBlcmltZW50IHRvIGFub3RoZXIsIGRlcGVuZGluZyBvbiB0aGUgb3ZlcmFsbCBleHBlcmltZW50YWwgY29uZGl0aW9ucy4gVGhpcyBpcyBjb21tb24gYW1vbmcgYWxsIGV4cGVyaW1lbnRzIGluIHdoaWNoIGNoYXJnZWQgIHN1cmZhY2VzIGFyZSBwcmVzZW50LCB3aGVyZWFzIHRoZSBhYnNvbHV0ZSB2YWx1ZSBkZXBlbmRzIG1vcmUgb24gdGhlIGV4cGVyaW1lbnRhbCBjb25kaXRpb25zLiBSZWdhcmRsZXNzIG9mICB0aGUgYWJzb2x1dGUgdmFsdWUgb2YgzrYsIHRoZSBjaGFyZ2VkIE1OQnMgd2lsbCBiZSByZXBlbGxpbmcgZWFjaCBvdGhlciwgdGh1cyBwcmV2ZW50aW5nIGJ1YmJsZSBjb2FsZXNjZW5jZS4gIFRoaXMgaGVscHMgdG8gcmV0YWluIGJ1YmJsZXMgaW4gdGhlIHNvbHV0aW9uIGZvciBhIGxvbmdlciB0aW1lLiBOZWdhdGl2ZWx5IGNoYXJnZWQgYnViYmxlcywgd2l0aCB0aGVpciBsb25nZXZpdHkgIGluIHdhdGVyLCBhc3Npc3QgaW4gdGhlIHRyZWF0bWVudCBwcm9jZXNzLiBJbiBhZGRpdGlvbiwgaGlnaCBETyBjb250ZW50cyBpbiB3YXRlciBmYWNpbGl0YXRlIHByb2Nlc3NlcyBzdWNoIGFzIEZpZy4gOC4gKGNvbnRpbnVlZCkgRmlnLiA5LiBETyBjb25jZW50cmF0aW9uIG9mIFNNQnMgYXQgZGlmZmVyZW50IGFlcmF0aW9uIHRpbWVzIGFuZCB2b2x1bWVzIGFuZCB0aGVpciBsb25nZXZpdHkgYWZ0ZXIgZ2VuZXJhdG9yIHN0b3BwYWdlLiBhZXJhdGlvbiwgd2hpY2ggcmVxdWlyZSBhIGhpZ2ggb3h5Z2VuIGNvbnRlbnQuIEFjY29yZGluZ2x5LCB0aGUgdXNlIG9mIE1Ccy9TTUJzL05CcyBpcyBtb3JlIGJlbmVmaWNpYWwgdGhhbiBjb252ZW50aW9uYWwgcHJvY2Vzc2VzLiBUaGUgcGh5c2ljYWwgbWVjaGFuaXNtIG9mIGJ1bGsgTkIgc3RhYmlsaXR5IGhhcyBiZWVuIGV4cGxhaW5lZCBieSBaaGFuZyBldCBhbC4gKDIwMjApNjYuIENoYXJnZSBlbnJpY2htZW50IGlzIGNhdXNlZCBieSB0aGUgc2lnbmlmaWNhbnQgYWZmaW5pdHkgb2YgbmVnYXRpdmUgY2hhcmdlcyBmb3IgdGhlIE5CIGludGVyZmFjZSwgd2hpY2ggcmVzdWx0cyBpbiBlbGVjdHJpYyBmaWVsZCBlbmVyZ3kgdGhhdCBnaXZlcyByaXNlIHRvIGEgbG9jYWwgbWluaW11bSBpbiB0aGUgZnJlZS1lbmVyZ3kgY29zdCBvZiBidWJibGUgcHJvZHVjdGlvbiBhbmQgdGhlcm1vZHluYW1pYyBtZXRhc3RhYmlsaXR5ICBvZiB0aGUgY2hhcmdlZCBOQnMuIEEgc2l6ZS1kZXBlbmRlbnQgZm9yY2UgaXMgZ2VuZXJhdGVkIG1lY2hhbmljYWxseSBieSBleGNlc3Mgc3VyZmFjZSBjaGFyZ2VzLiBUaGlzIGZvcmNlIHNlcnZlcyAgYXMgYSByZXN0b3JpbmcgZm9yY2Ugd2hlbiB0aGUgTkIgaXMgdGhlcm1vZHluYW1pY2FsbHkgZGlzdHVyYmVkIGZyb20gaXRzIGVxdWlsaWJyaXVtIHN0YXRlIGFuZCBiYWxhbmNlcyB0aGUgIExhcGxhY2UgcHJlc3N1cmUuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjg1LjcwOTg2MTc1NTM3MTEsImxlZnQiOjI2LjE5MDQ1NjM5MDM4MDg2LCJ3aWR0aCI6NjcuMjU1MDY1OTE3OTY4NzUsImhlaWdodCI6MS41NTE4MzgwNDAzNTE4Njc3fX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjg2Ljk4ODA0NDczODc2OTUzLCJsZWZ0IjoyNi4xOTA0NTYzOTAzODA4Niwid2lkdGgiOjY3LjI1NDMxMDYwNzkxMDE2LCJoZWlnaHQiOjEuNTUxODM4MDQwMzUxODY3N319LHsicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjo4OC4yNjYyMjAwOTI3NzM0NCwibGVmdCI6MjYuMTkwNDU2MzkwMzgwODYsIndpZHRoIjo2Ny4yNDkxNjA3NjY2MDE1NiwiaGVpZ2h0IjoxLjU1MTgzODA0MDM1MTg2Nzd9fSx7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6ODkuNTQ0NDAzMDc2MTcxODgsImxlZnQiOjI2LjE5MDQ1NjM5MDM4MDg2LCJ3aWR0aCI6NjcuMjUwNTY0NTc1MTk1MzEsImhlaWdodCI6MS41NTE4MzgwNDAzNTE4Njc3fX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjkwLjgyMjU4NjA1OTU3MDMxLCJsZWZ0IjoyNi4xOTA0NTYzOTAzODA4Niwid2lkdGgiOjY3LjI0MTE5NTY3ODcxMDk0LCJoZWlnaHQiOjEuNTUxODM4MDQwMzUxODY3N319LHsicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjo5Mi4xMDA3NjkwNDI5Njg3NSwibGVmdCI6MjYuMTkwNDU2MzkwMzgwODYsIndpZHRoIjo2Ny4yNDI4NDM2Mjc5Mjk2OSwiaGVpZ2h0IjoxLjU1MTgzODA0MDM1MTg2Nzd9fSx7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6NDEuODIwMjY2NzIzNjMyODEsImxlZnQiOjI2LjE5MDQ1NjM5MDM4MDg2LCJ3aWR0aCI6MTEuNzgzNTc0MTA0MzA5MDgyLCJoZWlnaHQiOjEuNTkyMTAwOTc3ODk3NjQ0fX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjc4LjE4Mjc3NzQwNDc4NTE2LCJsZWZ0IjoyNi4xOTA0NTYzOTAzODA4Niwid2lkdGgiOjY2LjA4NDUwMzE3MzgyODEyLCJoZWlnaHQiOjIuODcwMjgxNjk2MzE5NTh9fSx7InBhZ2VOdW1iZXIiOjExLCJiYm94Ijp7InRvcCI6NzQuMjg4OTAyMjgyNzE0ODQsImxlZnQiOjI2LjE5MDQ1NjM5MDM4MDg2LCJ3aWR0aCI6NjcuMjQ0NTA2ODM1OTM3NSwiaGVpZ2h0IjoyLjgzMDAxODc1ODc3MzgwMzd9fSx7InBhZ2VOdW1iZXIiOjExLCJiYm94Ijp7InRvcCI6NzYuNzg5Njk1NzM5NzQ2MSwibGVmdCI6MjguMjA2MzI3NDM4MzU0NDkyLCJ3aWR0aCI6NjUuMjMyMDU1NjY0MDYyNSwiaGVpZ2h0IjoxLjYwNzQwNzQ1MDY3NTk2NDR9fSx7InBhZ2VOdW1iZXIiOjExLCJiYm94Ijp7InRvcCI6NzguMTIzNDQzNjAzNTE1NjIsImxlZnQiOjI2LjE5MDQ1NjM5MDM4MDg2LCJ3aWR0aCI6NjcuMjUzNDAyNzA5OTYwOTQsImhlaWdodCI6MS41NTE4MzgwNDAzNTE4Njc3fX0seyJwYWdlTnVtYmVyIjoxMSwiYmJveCI6eyJ0b3AiOjc5LjQwMTYyNjU4NjkxNDA2LCJsZWZ0IjoyNi4xOTA0NTYzOTAzODA4Niwid2lkdGgiOjY3LjI1MDIyODg4MTgzNTk0LCJoZWlnaHQiOjEuNTUxODM4MDQwMzUxODY3N319LHsicGFnZU51bWJlciI6MTEsImJib3giOnsidG9wIjo4MC42Nzk4MDE5NDA5MTc5NywibGVmdCI6MjYuMTkwNDU2MzkwMzgwODYsIndpZHRoIjo2Ny4yNDU3MzUxNjg0NTcwMywiaGVpZ2h0IjoxLjU1MTgzODA0MDM1MTg2Nzd9fSx7InBhZ2VOdW1iZXIiOjExLCJiYm94Ijp7InRvcCI6ODEuOTU3OTg0OTI0MzE2NCwibGVmdCI6MjYuMTkwNDU2MzkwMzgwODYsIndpZHRoIjo2Ny4yNTE3Mzk1MDE5NTMxMiwiaGVpZ2h0IjoxLjU1MTgzODA0MDM1MTg2Nzd9fSx7InBhZ2VOdW1iZXIiOjExLCJiYm94Ijp7InRvcCI6ODMuMjM2MTYwMjc4MzIwMzEsImxlZnQiOjI2LjE5MDQ1NjM5MDM4MDg2LCJ3aWR0aCI6NjcuMjU2NzM2NzU1MzcxMSwiaGVpZ2h0IjoxLjU1MTgzODA0MDM1MTg2Nzd9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIxMSJ9fV0%3D.

Internally, nanobubbles experience elevated Laplace pressure, ΔP = 2σ/r, where σ is surface tension (~72 mN/m for water) and r is radius, often exceeding several atmospheres for 100 nm bubbles. This hyperbaric environment stabilizes the gas core against diffusive loss while promoting interfacial reactions, including the generation of reactive oxygen species such as hydroxyl radicals (•OH), superoxide (O₂⁻•), and hydrogen peroxide (H₂O₂) (Svetovoy, 2021)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlYuIEIuIiwiZmFtaWx5IjoiU3ZldG92b3kifX1dLCJpZHMiOnsiaXNzbiI6WyIxMzU5LTAyOTQiLCIxODc5LTAzOTkiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMTYvai5jb2Npcy4yMDIxLjEwMTQyMyIsIm1hZyI6IjMxMjY0ODQ4NDgiLCJvcGVuYWxleCI6IlczMTI2NDg0ODQ4In0sInRpdGxlIjoiU3BvbnRhbmVvdXMgY2hlbWljYWwgcmVhY3Rpb25zIGJldHdlZW4gaHlkcm9nZW4gYW5kIG94eWdlbiBpbiBuYW5vYnViYmxlcyIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2ouY29jaXMuMjAyMS4xMDE0MjMiLCJyZXNvdXJjZVVybCI6Imh0dHA6Ly9hcnhpdi5vcmcvcGRmLzIxMDIuMDMxMjYiLCJpc3N1ZWREYXRlIjoiMjAyMS0wMi0wNSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkN1cnJlbnQgT3BpbmlvbiBpbiBDb2xsb2lkICYgSW50ZXJmYWNlIFNjaWVuY2UiLCJ2b2x1bWUiOjUyfSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjoxMDE0MjMsImxhc3RQYWdlIjoxMDE0MjN9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRWxzZXZpZXIgQlYifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjYxLCJjaXRlZEJ5Q291bnQiOjJ9fSwicXVvdGUiOiJUaGlzIGF0dGVudGlvbiBpcyBkcml2ZW4gYnkgYSBmdW5kYS1tZW50YWwgcHJvYmxlbTogTkJzIGxpdmUgbXVjaCB0b28gbG9uZyBpbiBjb21wYXJpLXNvbiB3aXRoIHRoZSBwcmVkaWN0aW9uIGdpdmVuIGJ5IHRoZSBkaWZmdXNpb24gZGlzc29sdS10aW9uIHRoZW9yeSBbNCwgNV0uIE9uIHRoZSBvdGhlciBoYW5kLCBleGNpdGluZyBhcHBsaWNhLXRpb25zIG9mIE5CcyBpbiBlbnZpcm9ubWVudCBbNiwgN10sIGFncmljdWx0dXJlIFs4XSwgZmxvdGEtdGlvbiBbOSwgMTBdLCBoZWFsdGhjYXJlIFsxMSwgMTJdLCBhbmQgY2hlbWlzdHJ5IFsxM10gaGF2ZSBlbWVyZ2VkIHRvIG5hbWUgb25seSBhIGZldy4gSW4gY29udHJhc3Qgd2l0aCBzdXJmYWNlIE5Ccywgd2hpY2ggZXhpc3Qgb24gdGhlIHNvbGlkLWxpcXVpZCBpbnRlcmZhY2UsIGJ1bGsgTkJzIGZpbGwgdGhlIHZvbHVtZSBvZiBsaXF1aWQuIFRoZXNlIE5CcyBhcmUgZ2VuZXJhdGVkIGJ5IG1vc3RseSBtZWNoYW5pY2FsIGluamVjdGlvbiBvZiBkaWYtZmVyZW50IGdhc2VzIFs5XSBvciBieSB1bHRyYXNvbmljIGNhdml0YXRpb24gWzEwXSwgYnV0IGh5LWRyb2dlbiBhbmQgb3h5Z2VuIE5CcyB3aGVyZSBhbHNvIGdlbmVyYXRlZCBieSBEQyBlbGVjdHJvbHlzaXMgWzE0LCAxNV0uIExhdGVyIHRoZXNlIG1ldGhvZHMgd2lsbCBiZSBjYWxsZWQgdGhlIHJlZ3VsYXIgbWV0aG9kcy4gVGhlIHNpemUgZGlzdHJpYnV0aW9uIG9mIHRoZSBOQnMgd2FzIGNoYXJhY3Rlcml6ZWQgYnkgdGhlIGR5bmFtaWMgbGlnaHQgc2NhdHRlcmluZyBhbmQgaW5kaXZpZHVhbCBidWJibGVzIHdlcmUgb2JzZXJ2ZWQgYnkgZWxlY3Ryb24gbWljcm9zY29weSBmcm9tIGZyZWV6ZSBmcmFjdHVyZSByZXBsaWNhcyBbMTZdLCBieSBwaGFzZSBtaWNyb3Njb3B5IGFuZCBwb2xhcmltZXRyaWMgc2NhdHRlcm9tZXRyeSBbMTddLiBVbmV4cGVjdGVkbHksIE5CcyBjb250YWluaW5nIG94eWdlbiBhcmUgYWJsZSB0byBwcm8tZHVjZSByZWFjdGl2ZSBveHlnZW4gc3BlY2llcyB3aGljaCBjYW4gb3hpZGl6ZSBwb2xsdS10YW50cyBhbmQgcGF0aG9nZW5zIGluIHdhdGVyIFsxOOKAkzIwXSwgd2hpbGUgc2VwYXJhdGUgb3h5LWdlbiBtb2xlY3VsZXMgaGF2ZSBubyBlZmZlY3Qgb24gb3JnYW5pY3MuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6MzYuOTUwNjQ1NDQ2Nzc3MzQ0LCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA1MjI2MTM1MjUzOTA2LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjM4LjM5NzIzNTg3MDM2MTMzLCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA0ODk5OTc4NjM3Njk1LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjM5Ljg0MzgyNjI5Mzk0NTMxLCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA1MjI2MTM1MjUzOTA2LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjQxLjI5MDQxNjcxNzUyOTMsImxlZnQiOjguODIzNTI1NDI4NzcxOTczLCJ3aWR0aCI6NDAuMDUzODgyNTk4ODc2OTUsImhlaWdodCI6MS4yNTc5MDU5NjAwODMwMDc4fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NDIuNzM3MDEwOTU1ODEwNTUsImxlZnQiOjguODIzNTI1NDI4NzcxOTczLCJ3aWR0aCI6NDAuMDUyMjQ2MDkzNzUsImhlaWdodCI6MS4yNTc5MDU5NjAwODMwMDc4fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NDQuMTgzNjAxMzc5Mzk0NTMsImxlZnQiOjguODIzNTI1NDI4NzcxOTczLCJ3aWR0aCI6NDAuMDUyMjQ2MDkzNzUsImhlaWdodCI6MS4yNTc5MDU5NjAwODMwMDc4fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NDUuNjMwMTg3OTg4MjgxMjUsImxlZnQiOjguODIzNTI1NDI4NzcxOTczLCJ3aWR0aCI6MjAuNTMyMzk4MjIzODc2OTUzLCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjQ3LjA3Njc4MjIyNjU2MjUsImxlZnQiOjEwLjQ1MTQ2NjU2MDM2Mzc3LCJ3aWR0aCI6MzguNDE5NDk4NDQzNjAzNTE2LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjQ4LjUyMzM3MjY1MDE0NjQ4NCwibGVmdCI6OC44MjM1MjU0Mjg3NzE5NzMsIndpZHRoIjo0MC4wNTM4ODY0MTM1NzQyMiwiaGVpZ2h0IjoxLjI1NzkwNTk2MDA4MzAwNzh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo0OS45Njk5NjY4ODg0Mjc3MzQsImxlZnQiOjguODIzNTI1NDI4NzcxOTczLCJ3aWR0aCI6NDAuMDQ3Mzc0NzI1MzQxOCwiaGVpZ2h0IjoxLjI1NzkwNTk2MDA4MzAwNzh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo1MS40MTY1NTczMTIwMTE3MiwibGVmdCI6OC44MjM1MjU0Mjg3NzE5NzMsIndpZHRoIjo0MC4wNTIyNDYwOTM3NSwiaGVpZ2h0IjoxLjI1NzkwNTk2MDA4MzAwNzh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo1Mi44NjMxNDM5MjA4OTg0NCwibGVmdCI6OC44MjM1MjU0Mjg3NzE5NzMsIndpZHRoIjo0MC4wNDczNzQ3MjUzNDE4LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjU0LjMwOTczODE1OTE3OTY5LCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA0NDExMzE1OTE3OTY5LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjU1Ljc1NjMzMjM5NzQ2MDk0LCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA0OTAxMTIzMDQ2ODc1LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjU3LjIwMjkxOTAwNjM0NzY1NiwibGVmdCI6OC44MjM1MjU0Mjg3NzE5NzMsIndpZHRoIjo0MC4wNDczNzQ3MjUzNDE4LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjU4LjY0OTUwOTQyOTkzMTY0LCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA0OTAwMzYwMTA3NDIyLCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjYwLjA5NjEwMzY2ODIxMjg5LCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA0NTc0NTg0OTYwOTM3NSwiaGVpZ2h0IjoxLjI1NzkwNTk2MDA4MzAwNzh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2MS41NDI2OTAyNzcwOTk2MSwibGVmdCI6OC44MjM1MjU0Mjg3NzE5NzMsIndpZHRoIjoyNS43MTIzMDMxNjE2MjEwOTQsImhlaWdodCI6MS4yNTc5MDU5NjAwODMwMDc4fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NjIuOTg5NDA2NTg1NjkzMzYsImxlZnQiOjEwLjQ1MTQ2NjU2MDM2Mzc3LCJ3aWR0aCI6MzguNDIxMTEyMDYwNTQ2ODc1LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjY0LjQzNTk5NzAwOTI3NzM0LCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA0NTc0NTg0OTYwOTM3NSwiaGVpZ2h0IjoxLjI1NzkwNTk2MDA4MzAwNzh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2NS44ODI1OTEyNDc1NTg2LCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA0ODk5NTk3MTY3OTY5LCJoZWlnaHQiOjEuMjU3OTAyMTQ1Mzg1NzQyMn19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjY3LjMyOTE5MzExNTIzNDM4LCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjMwLjY2OTE3OTkxNjM4MTgzNiwiaGVpZ2h0IjoxLjI1NzkwMjE0NTM4NTc0MjJ9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIxMDE0MjMifX1d. ROS production stems from localized cavitation-like dynamics during bubble oscillation or charge-induced bond breaking at the interface, enabling oxidative degradation of recalcitrant organics and disinfection without bulk heating. In oxygen nanobubbles, this yields concentrations of 10⁹-10¹¹ particles/mL, with ROS fluxes sufficient to reduce ammonia by up to 9% in shrimp ponds (Nghĩa et al., 2022; Susanti et al., 2021)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5ndXnhu4VuIEjhu691IiwiZmFtaWx5IjoiTmdoxKlhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5ndXllbiBUaGkiLCJmYW1pbHkiOiJOZ3V5ZW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUGhhbiBUcuG7jW5nIiwiZmFtaWx5IjoiQsOsbmgifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTGUgVGhpIiwiZmFtaWx5IjoiTWF5In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRvbmcgVHJhbiIsImZhbWlseSI6Ikh1eSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQaOG6oW0gVGjDoWkiLCJmYW1pbHkiOiJHaWFuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTb3BoaWUiLCJmYW1pbHkiOiJTdOKAkEhpbGFpcmUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUGhhbiBUaGkiLCJmYW1pbHkiOiJWYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMjM0LTE3NDkiLCIyMjM0LTE3NTciXSwiaXNibiI6W10sImRvaSI6IjEwLjQ3ODUzL2Zhcy4yMDIyLmUzOSIsIm9wZW5hbGV4IjoiVzQyOTQ1OTc1NTQifSwidGl0bGUiOiJFZmZlY3Qgb2YgbmFub2J1YmJsZXMgKG94eWdlbiwgb3pvbmUpIG9uIHRoZSBQYWNpZmljIHdoaXRlIHNocmltcCAoUGVuYWV1cyB2YW5uYW1laSksIFZpYnJpbyBwYXJhaGFlbW9seXRpY3VzIGFuZCB3YXRlciBxdWFsaXR5IHVuZGVyIGxhYiBjb25kaXRpb25zIiwiYWJzdHJhY3QiOiJUaGlzIHN0dWR5IGFzc2Vzc2VkIHRoZSBlZmZlY3RzIG9mIG94eWdlbiBhbmQgb3pvbmUgbmFub2J1YmJsZXMgb24gZ2lsbCBtb3JwaG9sb2d5LCB3ZWlnaHQgZ2FpbiwgYW5kIG1vcnRhbGl0eSBvZiBQYWNpZmljIHdoaXRlIHNocmltcCAoUGVuYWV1cyB2YW5uYW1laSksIGFzIHdlbGwgYXMgdGhlIGxldmVsIG9mIFZpYnJpbyBwYXJhaGFlbW9seXRpY3VzIGFuZCB3YXRlciBxdWFsaXR5IG9mIHNocmltcCBjdWx0dXJlIHRhbmtzIHVuZGVyIGxhYiBjb25kaXRpb25zLiBUd28gZXhwZXJpbWVudHMgd2VyZSBjYXJyaWVkIG91dCB3aXRoIG94eWdlbiBtYWNyb2J1YmJsZSwgb3pvbmUgbWFjcm9idWJibGUsIG94eWdlbiBuYW5vYnViYmxlLCBvem9uZSBuYW5vYnViYmxlLCBhbmQgY29udHJvbCB0cmVhdG1lbnRzIChhaXItc3RvbmUgbWFjcm9idWJibGUpLiBFeHBlcmltZW50cyB3ZXJlIGRvbmUgaW4gdHJpcGxpY2F0ZSBpbiAxMDAgTCB0YW5rcyB3aXRoIDE14oCwIHNhbGluZSB3YXRlciwgYW5kIDIwIHNocmltcCBwZXIgdGFuay4gVGFua3MgaW4gRXhwZXJpbWVudCAxIHdlcmUgbm90IGlub2N1bGF0ZWQgd2l0aCBiYWN0ZXJpYTsgdGFua3MgaW4gRXhwZXJpbWVudCAyIHdlcmUgaW5vY3VsYXRlZCB3aXRoIFYuIHBhcmFoYWVtb2x5dGljdXMgYXQgYSBjb25jZW50cmF0aW9uIG9mIDEwNiBDRlUvbUwuIFRoZSByZXN1bHRzIHJldmVhbGVkIHRoYXQgc2hvcnQgdHJlYXRtZW50cyB3aXRoIG96b25lIG5hbm9idWJibGVzIGhhZCBtaW5pbWFsIGltcGFjdCBvbiBzaHJpbXAgZ2lsbHMsIG1vcnRhbGl0eSwgYW5kIGdyb3d0aCByYXRlcywgcmVkdWNlZCBWLiBwYXJhaGFlbW9seXRpY3VzIGNvbmNlbnRyYXRpb24gaW4gd2F0ZXIgY29tcGFyZWQgdG8gdGhlIG90aGVyIGdyb3VwcywgYW5kIGltcHJvdmUgd2F0ZXIgcXVhbGl0eS4gVGhlc2UgbGFib3JhdG9yeSByZXN1bHRzIGluZGljYXRlIHRoYXQgb3pvbmUgbmFub2J1YmJsZSB0cmVhdG1lbnQgbWF5IGJlIHVzZWZ1bCBmb3IgY29udHJvbGxpbmcgVi4gcGFyYWhhZW1vbHl0aWN1cy4gTW9yZSB3b3JrIGlzIG5lZWRlZCB0byBmaW5kIHRoZSBiZXN0IHByb3RvY29sIHRvIGFwcGx5IHRoZSB0ZWNobm9sb2d5IG9uIGEgY29tbWVyY2lhbCBzY2FsZS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuNDc4NTMvZmFzLjIwMjIuZTM5IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5lLWZhcy5vcmcvZG93bmxvYWQvZG93bmxvYWRfcGRmP3BpZD1mYXMtMjUtOC00MjkiLCJpc3N1ZWREYXRlIjoiMjAyMi0wOC0wMSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZpc2hlcmllcyBhbmQgYXF1YXRpYyBzY2llbmNlcyIsImlzc3VlIjo4LCJ2b2x1bWUiOjI1LCJpbXBhY3RGYWN0b3IiOiIxLjM0In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6NDI5LCJsYXN0UGFnZSI6NDQwfX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlNwcmluZ2VyIFNjaWVuY2UrQnVzaW5lc3MgTWVkaWEifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjMwLCJjaXRlZEJ5Q291bnQiOjEzfX0sInF1b3RlIjoiVGhlIGluZHVzdHJ5IHRoZXJlZm9yZSBuZWVkcyB0byBmaW5kIGFsdGVybmF0aXZlIG1lYXN1cmVzICB0byBwcmV2ZW50IGFuZCBjb250cm9sIHRoaXMgcGF0aG9nZW4uIE5hbm9idWJibGUgdGVjaG5vbG9neSBwcm9kdWNlcyBzbWFsbCBnYXMgYnViYmxlcyB3aXRoIGEgZGlhbWV0ZXIgb2YgbGVzcyB0aGFuIDIwMCBubSAoQWdyYXdhbCBldCBhbC4sIDIwMTE7IENoYXBsaW4sIDIwMjEpIHdoaWNoIGNhbiBleGlzdCBpbiB0aGUgd2F0ZXIgY29sdW1uIGZvciB3ZWVrcyAoQXpldmVkbyAgZXQgYWwuLCAyMDE2OyBQYXJtYXIgJiBNYWp1bWRlciwgMjAxMykuIFRoaXMgcGhlbm9tZW5vbiBpcyAgYXNzb2NpYXRlZCB3aXRoIHRoZSBwaHlzaWNhbCBjaGFyYWN0ZXJpc3RpYyBvZiB0aGVzZSBzbWFsbCBidWItYmxlcyAoV2VpanMgJiBMb2hzZSwgMjAxMykgYW5kIGhpZ2ggemV0YSBwb3RlbnRpYWwgdmFsdWUgd2hpY2ggIGtlZXBzIHRoZW0gZnJvbSBhZ2dyZWdhdGluZyBpbnRvIGxhcmdlciBidWJibGVzIChLaXJieSwgMjAxMDsgIFNqb2dyZWVuIGV0IGFsLiwgMjAxOCkuIE5hbm9idWJibGUgdGVjaG5vbG9neSBoYXMgYSBsb3Qgb2YgcHJvbS1pc2luZyBlZmZlY3RpdmUgYXBwbGljYXRpb24gaW4gYXF1YWN1bHR1cmUgYmVjYXVzZSBvZiBpdHMgYWJpbGl0eSAgdG8gaW5jcmVhc2UgYW5kIG1haW50YWluIG94eWdlbiBjb250ZW50IGluIHdhdGVyIGFuZCBjb250cm9sICBiYWN0ZXJpYSwgYSBwb3NzaWJsZSBtZWFzdXJlIHRvIHJlZHVjZSB0aGUgdXNlIG9mIGFudGliaW90aWNzLiBPeHlnZW4gbmFub2J1YmJsZXMgb3B0aW1pemUgdGhlIGRpc3NvbHZlZCBveHlnZW4gaW4gYXF1YXRpYyBzeXN0ZW1zIChHYWxhbmcgZXQgYWwuLCAyMDE5KS4gSGlnaCBsZXZlbHMgb2Ygb3h5Z2VuIG1heSBzcGVlZCB1cCBtZXRhYm9saXNtLCBwcm9tb3RlIGhpZ2hlciBmb29kIGludGFrZSwgYW5kIGluY3JlYXNlICBncm93dGggb2YgZmFybWVkIHNocmltcCAoRWJpbmEgZXQgYWwuLCAyMDEzKS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNS4wNjg2NDU0NzcyOTQ5MiwibGVmdCI6OC4wOTUyMzU4MjQ1ODQ5NjEsIndpZHRoIjo0MS4wNzIyMTIyMTkyMzgyOCwiaGVpZ2h0IjoxLjY5OTYzNTE0ODA0ODQwMDl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNi44MzI1MzA5NzUzNDE4LCJsZWZ0Ijo4LjA5NTIzNTgyNDU4NDk2MSwid2lkdGgiOjIzLjYwNDE4NTEwNDM3MDExNywiaGVpZ2h0IjoxLjY5OTYzNTE0ODA0ODQwMDl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC41OTY0MjAyODgwODU5NCwibGVmdCI6MTAuOTUyMzY0OTIxNTY5ODI0LCJ3aWR0aCI6MzguMjE1ODc3NTMyOTU4OTg0LCJoZWlnaHQiOjEuNjk5NjM1MTQ4MDQ4NDAwOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQwLjM2MDMwOTYwMDgzMDA4LCJsZWZ0Ijo4LjA5NTIzNTgyNDU4NDk2MSwid2lkdGgiOjQxLjA3Njc1OTMzODM3ODkwNiwiaGVpZ2h0IjoxNy41NzQ2MzA3MzczMDQ2ODh9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1Ny45OTkxOTUwOTg4NzY5NSwibGVmdCI6MTAuOTUyMzY0OTIxNTY5ODI0LCJ3aWR0aCI6MzguMjAxNDQ2NTMzMjAzMTI1LCJoZWlnaHQiOjEuNjk5NjM1MTQ4MDQ4NDAwOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjU5Ljc2MzA4MDU5NjkyMzgzLCJsZWZ0Ijo4LjA5NTIzNTgyNDU4NDk2MSwid2lkdGgiOjQxLjA2MzMyNzc4OTMwNjY0LCJoZWlnaHQiOjEuNjk5NjM1MTQ4MDQ4NDAwOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjYxLjUyNjk3MzcyNDM2NTIzNCwibGVmdCI6OC4wOTUyMzU4MjQ1ODQ5NjEsIndpZHRoIjo0MS4wNjY4NDQ5NDAxODU1NSwiaGVpZ2h0IjoxLjY5OTYzNTE0ODA0ODQwMDl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2My4yOTA4NjMwMzcxMDkzNzUsImxlZnQiOjguMDk1MjM1ODI0NTg0OTYxLCJ3aWR0aCI6NDAuNzE0MjkwNjE4ODk2NDg0LCJoZWlnaHQiOjEuNjk5NjMxMzMzMzUxMTM1M319XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjQzMCJ9fSx7Im9yaWdpbiI6NCwid29yayI6eyJ0eXBlIjo4LCJzdWJ0eXBlIjp7Im9uZW9mS2luZCI6ImFydGljbGVUeXBlIiwiYXJ0aWNsZVR5cGUiOjF9LCJjb250cmlidXRvcnMiOlt7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJMaWx5IiwiZmFtaWx5IjoiU3VzYW50aSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTdXl1ZCBXYXJubyIsImZhbWlseSI6IlV0b21vIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5vdmVyaXRhIERpYW4iLCJmYW1pbHkiOiJUYWthcmluYSJ9fV0sImlkcyI6eyJpc3NuIjpbIjIxMTctNDQ1OCIsIjIyNzMtMTcwOSJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMTA1MS9iaW9jb25mLzIwMjEzMzA1MDA1IiwibWFnIjoiMzE5MzU2MzcwMyIsIm9wZW5hbGV4IjoiVzMxOTM1NjM3MDMifSwidGl0bGUiOiJTdXN0YWluYWJpbGl0eSBhbmQgZmVhc2liaWxpdHkgYXNzZXNzbWVudHMgb2YgbmFub2J1YmJsZSBhZXJhdGlvbiB0ZWNobm9sb2d5IGluIGVjb25vbWljLXNvY2lvIGVudmlyb25tZW50IG9mIFBlbmFldXMgdmFubmFtZWkgc2hyaW1wIGZhcm1pbmciLCJhYnN0cmFjdCI6IlBlbmFldXMgdmFubmFtZWkgc2hyaW1wIGZhcm1pbmcgaW4gSW5kb25lc2lhIGlzIGZhY2VkIHdpdGggc2V2ZXJhbCBjaGFsbGVuZ2VzIGluIHRoZSBlbnZpcm9ubWVudGFsLCBlY29ub21pYywgYW5kIHNvY2lhbCBhc3BlY3RzLiBUaGVyZWZvcmUsIHRoaXMgc3R1ZHkgYWltcyB0byBhc3Nlc3MgdGhlIGJlbmVmaXRzIG9mIG5vdmVsIG5hbm9idWJibGUgYWVyYXRpb24gc3lzdGVtcyBmb3IgdGhlIGVjb3N5c3RlbSwgYnVzaW5lc3NlcywgYW5kIGNvbW11bml0aWVzLiBUaGlzIGlzIGFuIGV4cGVyaW1lbnRhbCBzdHVkeSBjb25kdWN0ZWQgYnkgY29tcGFyaW5nIFAuIHZhbm5hbWVpIHBvc3QgbGFydmEgMTAgcmVhcmVkIGF0IGEgZGVuc2l0eSBvZiAyMDAwIGluZHMuL0wgaW4gYSBwb25kIHRyZWF0ZWQgd2l0aCBuYW5vYnViYmxlIGFuZCB3aXRob3V0IHRyZWF0bWVudHMgKGNvbnRyb2wpLiBGdXJ0aGVybW9yZSwgdGhlIE5ldCBQcmVzZW50IFZhbHVlIChOUFYpIGFuZCBwYXliYWNrIHBlcmlvZCB3ZXJlIGNhbGN1bGF0ZWQgdG8gYXNzZXNzIHRoZSBlY29ub21pYyBmZWFzaWJpbGl0eSBvZiBuYW5vYnViYmxlLCB3aGlsZSBjb21tdW5pdHkgaW50ZXJ2aWV3cyB3ZXJlIHVzZWQgZm9yIHRoZSBzb2NpYWwgc2VjdG9ycy4gQmFzZWQgb24gdGhlIHJlc3VsdHMsIGVudmlyb25tZW50YWwgd2FzdGVzIG9mIHNocmltcCBmYXJtaW5nIGluIGZvcm0gb2YgdG90YWwgYW1tb25pYSBuaXRyb2dlbiAoVEFOKSB3ZXJlIHJlZHVjZWQgYnkgOSUgZnJvbSAyLjU4IG1nL2wgKDk1JUNJOiAwLDkxLCA0LDI1KSBpbiBjb250cm9sIGNvbXBhcmVkIHRvIDIuMzUgbWcvbCAoOTUlQ0k6IDAsODYsIDMsODQpIGluIHRyZWF0bWVudC4gRnVydGhlcm1vcmUsIHRoZSByZXZlbnVlcyBmcm9tIHBvc3QgbGFydmEgc2FsZXMgZm9yIGZpdmUgeWVhcnMgYW5kIG5hbm9idWJibGUgaW52ZXN0bWVudCBjb3N0cyBzaG93ZWQgdGhhdCB0aGUgZXN0aW1hdGVkIE5QViB3YXMgSURSIDY0LDgyNCwzNzQgd2l0aCBhIHBheWJhY2sgcGVyaW9kIG9mIDEuNyB5ZWFycy4gVGhlIGludGVydmlld3Mgb24gdHJhZGl0aW9uYWwgc2hyaW1wIGZhcm1pbmcgc2hvd2VkIHRoYXQgNjEuMSUgdG8gNzIuMiUgb2YgY29tbXVuaXR5IG1lbWJlcnMgYWdyZWVkIG9uIHRoZSB1c2Ugb2YgbmFub2J1YmJsZSBhZXJhdGlvbiB0byBzdXBwb3J0IGxpdmVsaWhvb2RzLiBUaGVyZWZvcmUsIHRoZSB1c2Ugb2YgbmFub2J1YmJsZXMgaXMgZmVhc2libGUgdG8gc3VwcG9ydCBzdXN0YWluYWJsZSBQLiB2YW5uYW1laSBmYXJtaW5nLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDUxL2Jpb2NvbmYvMjAyMTMzMDUwMDUiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vd3d3LmJpby1jb25mZXJlbmNlcy5vcmcvYXJ0aWNsZXMvYmlvY29uZi9wZGYvMjAyMS8wNS9iaW9jb25mX2ljYXZlc3MyMDIxXzA1MDA1LnBkZiIsImlzc3VlZERhdGUiOiIyMDIxLTAxLTAxIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0wOSIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiQklPIFdlYiBvZiBDb25mZXJlbmNlcyIsInZvbHVtZSI6MzMsImltcGFjdEZhY3RvciI6IjAuNjIifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjo1MDA1LCJsYXN0UGFnZSI6NTAwNX19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJFRFAgU2NpZW5jZXMifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjExLCJjaXRlZEJ5Q291bnQiOjR9fSwicXVvdGUiOiJBYnN0cmFjdC4gUGVuYWV1cyB2YW5uYW1laSBzaHJpbXAgZmFybWluZyBpbiBJbmRvbmVzaWEgaXMgZmFjZWQgd2l0aCBzZXZlcmFsIGNoYWxsZW5nZXMgaW4gdGhlIGVudmlyb25tZW50YWwsIGVjb25vbWljLCBhbmQgc29jaWFsIGFzcGVjdHMuICBUaGVyZWZvcmUsIHRoaXMgc3R1ZHkgYWltcyB0byBhc3Nlc3MgdGhlIGJlbmVmaXRzIG9mIG5vdmVsIG5hbm9idWJibGUgIGFlcmF0aW9uIHN5c3RlbXMgZm9yIHRoZSBlY29zeXN0ZW0sIGJ1c2luZXNzZXMsIGFuZCBjb21tdW5pdGllcy4gVGhpcyBpcyBhbiAgZXhwZXJpbWVudGFsIHN0dWR5IGNvbmR1Y3RlZCBieSBjb21wYXJpbmcgUC4gdmFubmFtZWkgcG9zdCBsYXJ2YSAxMCAgcmVhcmVkIGF0IGEgZGVuc2l0eSBvZiAyMDAwIGluZHMuL0wgaW4gYSBwb25kIHRyZWF0ZWQgd2l0aCBuYW5vYnViYmxlIGFuZCAgd2l0aG91dCB0cmVhdG1lbnRzIChjb250cm9sKS4gRnVydGhlcm1vcmUsIHRoZSBOZXQgUHJlc2VudCBWYWx1ZSAoTlBWKSBhbmQgIHBheWJhY2sgcGVyaW9kIHdlcmUgY2FsY3VsYXRlZCB0byBhc3Nlc3MgdGhlIGVjb25vbWljIGZlYXNpYmlsaXR5IG9mICBuYW5vYnViYmxlLCB3aGlsZSBjb21tdW5pdHkgaW50ZXJ2aWV3cyB3ZXJlIHVzZWQgZm9yIHRoZSBzb2NpYWwgc2VjdG9ycy4gIEJhc2VkIG9uIHRoZSByZXN1bHRzLCBlbnZpcm9ubWVudGFsIHdhc3RlcyBvZiBzaHJpbXAgZmFybWluZyBpbiBmb3JtIG9mICB0b3RhbCBhbW1vbmlhIG5pdHJvZ2VuIChUQU4pIHdlcmUgcmVkdWNlZCBieSA5JSBmcm9tIDIuNTggbWcvbCAoOTUlQ0k6ICAwLDkxLCA0LDI1KSBpbiBjb250cm9sIGNvbXBhcmVkIHRvIDIuMzUgbWcvbCAoOTUlQ0k6IDAsODYsIDMsODQpIGluICB0cmVhdG1lbnQuIEZ1cnRoZXJtb3JlLCB0aGUgcmV2ZW51ZXMgZnJvbSBwb3N0IGxhcnZhIHNhbGVzIGZvciBmaXZlIHllYXJzIGFuZCAgbmFub2J1YmJsZSBpbnZlc3RtZW50IGNvc3RzIHNob3dlZCB0aGF0IHRoZSBlc3RpbWF0ZWQgTlBWIHdhcyBJRFIgIDY0LDgyNCwzNzQgd2l0aCBhIHBheWJhY2sgcGVyaW9kIG9mIDEuNyB5ZWFycy4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo0NS40ODMyODc4MTEyNzkzLCJsZWZ0IjoyMS44Mjc3NzAyMzMxNTQyOTcsIndpZHRoIjo1Ni44MDA1Njc2MjY5NTMxMjUsImhlaWdodCI6MS43NDkyMzg2MTAyNjc2MzkyfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NDcuMTY3MTcxNDc4MjcxNDg0LCJsZWZ0IjoyMS44Mjc3NzAyMzMxNTQyOTcsIndpZHRoIjo1Ni43NzUwMzk2NzI4NTE1NiwiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo0OC42MjM4MDk4MTQ0NTMxMjUsImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2Ljc3MTMwMTI2OTUzMTI1LCJoZWlnaHQiOjEuNDA2Nzg1OTY0OTY1ODIwM319LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjUwLjA4MDQ0ODE1MDYzNDc2NiwibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuODIwMjk3MjQxMjEwOTQsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NTEuNTM3MDkwMzAxNTEzNjcsImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2LjgyNjUzMDQ1NjU0Mjk3LCJoZWlnaHQiOjEuNDA2Nzg1OTY0OTY1ODIwM319LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjUzLjAxMDY2NTg5MzU1NDY5LCJsZWZ0IjoyMS44Mjc3NzAyMzMxNTQyOTcsIndpZHRoIjo1Ni43NjU3MjQxODIxMjg5MDYsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NTQuNDY3MzA0MjI5NzM2MzMsImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2Ljc5MzcxMjYxNTk2NjgsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NTUuOTIzOTQ2MzgwNjE1MjM0LCJsZWZ0IjoyMS44Mjc3NzAyMzMxNTQyOTcsIndpZHRoIjo1Ni43OTU2MTYxNDk5MDIzNDQsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NTcuMzgwNTg4NTMxNDk0MTQsImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2Ljc3NjkyNzk0Nzk5ODA1LCJoZWlnaHQiOjEuNDA2Nzg1OTY0OTY1ODIwM319LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjU4Ljg1NDE2NDEyMzUzNTE1NiwibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuNzgwNjY2MzUxMzE4MzYsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NjAuMzEwNzk4NjQ1MDE5NTMsImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2LjgzMzM3NDAyMzQzNzUsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NjEuNzY3NDQwNzk1ODk4NDQsImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2Ljc0NzAwOTI3NzM0Mzc1LCJoZWlnaHQiOjEuNDA2Nzg1OTY0OTY1ODIwM319LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjYzLjIyODMxMzQ0NjA0NDkyLCJsZWZ0IjoyMS44Mjc3NzAyMzMxNTQyOTcsIndpZHRoIjo1Ni43ODgxMjc4OTkxNjk5MiwiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2NC43MDE4ODkwMzgwODU5NCwibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuNzg2MjYyNTEyMjA3MDMsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NjYuMTU4NTMxMTg4OTY0ODQsImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2LjgxNTMxOTA2MTI3OTMsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNTAwNSJ9fV0%3D.

The mechanisms of action underscore nanobubbles' multifunctionality: enhanced mass transfer via Brownian motion and reduced rise velocity (Stokes' law: v propto r^2), ensuring prolonged liquid residence; ROS-mediated advanced oxidation processes that mineralize pollutants and disrupt microbial cell walls; and biophysical effects like altered interfacial tension influencing biofouling. These properties collectively amplify oxygen bioavailability, suppress pathogens (e.g., Vibrio spp.), and modulate microbial communities, outperforming macro-aeration by minimizing energy input and chemical use (Tekile et al., 2016)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFuZGluZXQgS2ViZWRlIiwiZmFtaWx5IjoiVGVraWxlIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikls4oCQSG8iLCJmYW1pbHkiOiJLaW0ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSmFpeWVvcCIsImZhbWlseSI6IkxlZSJ9fV0sImlkcyI6eyJpc3NuIjpbIjEyMjYtMTAyNSIsIjIwMDUtOTY4WCJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuNDQ5MS9lZXIuMjAxNi4wMjgiLCJtYWciOiIyNTIzNjY5NjU3Iiwib3BlbmFsZXgiOiJXMjUyMzY2OTY1NyJ9LCJ0aXRsZSI6IkV4dGVudCBhbmQgcGVyc2lzdGVuY2Ugb2YgZGlzc29sdmVkIG94eWdlbiBlbmhhbmNlbWVudCB1c2luZyBuYW5vYnViYmxlcyIsImFic3RyYWN0IjoiSW4gdGhpcyBzdHVkeSwgY2hhbmdlIGluIHdhdGVyLWRpc3NvbHZlZCBveHlnZW4gKERPKSB3YXMgYW5hbHl6ZWQgdW5kZXIgdmFyaW91cyBzeW50aGV0aWMgd2F0ZXIgcXVhbGl0aWVzIGFuZCBuYW5vYnViYmxlcyAoTkJzKSBhcHBsaWNhdGlvbiBjb25kaXRpb25zLCBzdWNoIGFzIGdhcyB0eXBlLCBpbml0aWFsIERPIGFzIHdlbGwgYXMgd2F0ZXIgZGlzc29sdmVkLCBzdXNwZW5kZWQgYW5kIG9yZ2FuaWMgbWF0dGVycyBjb250ZW50cy4gV2hlbiBveHlnZW4sIHJhdGhlciB0aGFuIGFpciwgd2FzIGludHJvZHVjZWQgaW50byBuaXRyb2dlbi1kZXNvcmJlZCB1bHRyYS1wdXJlIHdhdGVyLCB0aGUgc3RhZ25hdGlvbiB0aW1lIHdhcyBzaWduaWZpY2FudGx5IGluY3JlYXNlZC4gSXQgdG9vayB0ZW4gZGF5cyBmb3IgRE8gY29uY2VudHJhdGlvbiB0byBkcm9wIGJhY2sgdG8gc2F0dXJhdGlvbi4gVGhlIGhpZ2hlciB0aGUgaW5pdGlhbCBETyBjb25jZW50cmF0aW9uLCB0aGUgbG9uZ2VyIHBhcnRpY2xlcyB3ZXJlIG9ic2VydmVkIGFib3ZlIHNhdHVyYXRpb24gZHVlIHRvIHBhcnRpY2xlIHN0YWJpbGl0eSBpbXByb3ZlbWVudC4gVGhlIG94eWdlbiBtYXNzIHRyYW5zZmVyIHJhdGUgb2YgMC4wNDgyIG1nL0wvbWluIHdhcyBmb3VuZCB0byByZWFjaCBhIG1heGltdW0gYXQgYW4gZWxlY3Ryb2x5dGljIGNvbmNlbnRyYXRpb24gb2YgMC43NSBnL0wsIGJleW9uZCB3aGljaCB0aGUgdHJhbnNmZXIgcmF0ZSBkZWNyZWFzZWQgZHVlIHRvIGFkc29ycHRpb24gb2YgbmVnYXRpdmUgaW9ucyBvZiB0aGUgZWxlY3Ryb2x5dGUgYXQgdGhlIGludGVyZmFjZS4gSGlnaCBsZXZlbHMgb2YgdHVyYmlkaXR5IGNhdXNlZCBieSBzdXNwZW5kZWQgc29saWRzIGhhdmUgYmVjb21lIGEgYmFycmllciB0byBkaXNzb2x1dGlvbiBvZiBOQnMgb3h5Z2VuIGludG8gdGhlIHdhdGVyIHNvbHV0aW9uLCBhbmQgdGh1cyBhZmZlY3RlZCB0aGUgdHJhbnNmZXIgcGVyZm9ybWFuY2UuIE9uIHRoZSBvdGhlciBoYW5kLCBieSBhcHBseWluZyBOQnMgZm9yIGp1c3QgYW4gaG91ciwgdXAgdG8gNy4yJSBkZWdyYWRhdGlvbiBvZiBnbHVjb3NlIGFzIHJlcHJlc2VudGF0aXZlIG9yZ2FuaWMgbWF0dGVyIHdhcyBhY2hpZXZlZC4gVGh1cywgTkJzIHRlY2hub2xvZ3kgd291bGQgbWFpbnRhaW4gYSBoaWdoIERPIGV4dGVudCBmb3IgYW4gZXh0ZW5kZWQgZHVyYXRpb24sIGFuZCB0aHVzIGNhbiBpbXByb3ZlIHdhdGVyIHF1YWxpdHkgcHJvdmlkZWQgdGhhdCB3YXRlciBjaGVtaXN0cnkgaXMgY2xvc2VseSBtb25pdG9yZWQgZHVyaW5nIGl0cyBhcHBsaWNhdGlvbi4gS2V5d29yZHM6IEFwcGxpY2F0aW9uIGNvbmRpdGlvbnMsIERpc3NvbHZlZCBveHlnZW4sIE5hbm9idWJibGVzLCBXYXRlciBxdWFsaXR5IiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjQ0OTEvZWVyLjIwMTYuMDI4IiwicmVzb3VyY2VVcmwiOiJodHRwOi8vZWVlci5vcmcvdXBsb2FkL2Vlci0yMS00LTQyNy5wZGYiLCJpc3N1ZWREYXRlIjoiMjAxNi0wOS0xMiIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkVudmlyb25tZW50YWwgRW5naW5lZXJpbmcgUmVzZWFyY2giLCJpc3N1ZSI6NCwidm9sdW1lIjoyMSwiaW1wYWN0RmFjdG9yIjoiNC4wNSJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjQyNywibGFzdFBhZ2UiOjQzNX19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJLb3JlYW4gU29jaWV0eSBvZiBFbnZpcm9ubWVudGFsIEVuZ2luZWVyaW5nIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjoyNCwiY2l0ZWRCeUNvdW50IjozM319LCJxdW90ZSI6IlRoaXMgdmVyaWZpZXMgdGhlIHBvc3NpYmlsaXR5IG9mIHRoZSBleGlzdGVuY2UgIG9mIE5Ccywgc2luY2UgYXQgYSBoaWdoZXIgRE8gY29uY2VudHJhdGlvbiwgdGhlIG94eWdlbiBkaWZmdXNpb24gIHdvdWxkIHRha2UgbW9yZSB0aW1lLiBUaGUgaGlnaCBnYXMgY29uY2VudHJhdGlvbiBpbiB0aGUgbGlxdWlkICBleHBsYWlucyB0aGUgZXh0ZW5zaW9uIG9mIE5CIHN0YWJpbGl0eS4gIFRoZSBtYWduaXR1ZGVzIG9mIHpldGEgcG90ZW50aWFsIG9mIG94eWdlbiBuYW5vLWJ1YmJsZWQgd2F0ZXIgIHdlcmUgcG9zc2libHkgaGlnaGVyIHRoYW4gdGhvc2Ugb2YgYWlyIG5hbm8tYnViYmxlZCB3YXRlci4gQXQgIGEgaGlnaCBtYWduaXR1ZGUgemV0YSBwb3RlbnRpYWwgdmFsdWUsIHRoZSBlbGVjdHJpY2FsbHkgY2hhcmdlZCAgcGFydGljbGVzIHRlbmQgdG8gcmVwZWwgZWFjaCBvdGhlciwgYXZvaWRpbmcgYWdncmVnYXRpb24uIEZvciBidWJibGUgIGRpc3BlcnNpb24sIHRoZSBoaWdoIHpldGEgcG90ZW50aWFsIGNvdWxkIGNyZWF0ZSByZXB1bHNpb24gZm9yY2VzICB0aGF0IHdvdWxkIGF2b2lkIHRoZSBjb2FsZXNjZW5jZSBvZiBidWJibGVzLCB0aHVzIGNvbnRyaWJ1dGluZyAgdG8gdGhlIHN0YWJpbGl6YXRpb24gb2YgdGhlIGJ1YmJsZXMuIFRoZXJlZm9yZSwgdGhlIG94eWdlbiBOQnMgIGNvdWxkIGJlIHN0YWJpbGl6ZWQgaW4gd2F0ZXIgYnkgdGhlIGVsZWN0cmljYWxseSBjaGFyZ2VkIHN1cmZhY2VzLCAgd2hpbGUgdGhlIG5hbm8tYnViYmxlZCBhaXIgd291bGQgYmUgYmVsb3cgdGhlIGxpbWl0IG9mIHRoZSBzdGFiaWxpemEtdGlvbiBieSByZXB1bHNpb24gZm9yY2VzLiBGdXJ0aGVybW9yZSwgdGhlIGJpZyBkaWZmZXJlbmNlIGJldHdlZW4gdGhlIHBlcmlvZHMgaW4gd2hpY2ggIGJ1YmJsZXMgY291bGQgYmUgb2JzZXJ2ZWQgYXMgc3RhYmxlIGluIHRoZSBveHlnZW4gbmFuby1idWJibGVkICB3YXRlciAoMTAtMTUgZCkgYW5kIGluIHRoYXQgb2YgYWlyIG5hbm8tYnViYmxlZCAoMy01IGQpIGlzIGxpa2VseSBGaWcuIDYuIEdyYXBocyBzaG93aW5nIHN0YWduYXRpb24gdGltZSBmb3IgZGlmZmVyZW50IGluaXRpYWwgZGlzc29sdmVkIG94eWdlbiAoRE8pLiByZWxhdGVkIHRvIHRoZSBsb3dlciBkaXNzb2x1dGlvbiBvZiB0aGUgYWlyIGluIHdhdGVyIGluIHJlbGF0aW9uIHRvIHRoZSBkaXNzb2x1dGlvbiBvZiBveHlnZW4uICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6NjYuMTc4ODAyNDkwMjM0MzgsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6NDAuNzE0ODg5NTI2MzY3MTksImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo2Ny41NzY3NzQ1OTcxNjc5NywibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42ODgyNzA1Njg4NDc2NTYsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo2OC45NjcyMzE3NTA0ODgyOCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NTcxODA3ODYxMzI4MSwiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjcwLjM1NzY4ODkwMzgwODYsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6MjUuMzQxMjU1MTg3OTg4MjgsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3MS43NTU2NTMzODEzNDc2NiwibGVmdCI6OS41NDk2NDE2MDkxOTE4OTUsIndpZHRoIjozOS4wMjQ3ODQwODgxMzQ3NjYsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3My4xNDYxMDI5MDUyNzM0NCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NTQwMjk4NDYxOTE0MDYsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3NC41MzY1NjAwNTg1OTM3NSwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NTM0NjUyNzA5OTYwOTQsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3NS45MzQ1MzIxNjU1MjczNCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NzMyMzY4NDY5MjM4MywiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjc3LjMyNDk4OTMxODg0NzY2LCJsZWZ0Ijo3Ljg2NTU0NTI3MjgyNzE0OCwid2lkdGgiOjQwLjY0OTM1MzAyNzM0Mzc1LCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6NzguNzE1NDQ2NDcyMTY3OTcsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6NDAuNjU0NjI0OTM4OTY0ODQ0LCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODAuMTEzNDEwOTQ5NzA3MDMsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6NDAuNjUxMDUwNTY3NjI2OTUsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo4MS41MDM4NjA0NzM2MzI4MSwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NTQzNDI2NTEzNjcxOSwiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjgyLjg5NDMxNzYyNjk1MzEyLCJsZWZ0Ijo3Ljg2NTU0NTI3MjgyNzE0OCwid2lkdGgiOjQwLjMyMTgyMzEyMDExNzE5LCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODQuMjkyMjg5NzMzODg2NzIsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6MTUuODcwMjk1NTI0NTk3MTY4LCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODUuNjgyNzQ2ODg3MjA3MDMsImxlZnQiOjkuNTQ5NjQxNjA5MTkxODk1LCJ3aWR0aCI6MzguOTg5NzA0MTMyMDgwMDgsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo4Ny4wNzMyMDQwNDA1MjczNCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC43MTg0OTQ0MTUyODMyLCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODguNDcxMTY4NTE4MDY2NCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42ODc0MTk4OTEzNTc0MiwiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjM0LjUxMzI3MTMzMTc4NzExLCJsZWZ0Ijo1MS45MzI3Nzc0MDQ3ODUxNTYsIndpZHRoIjo0MC42NDcwNTI3NjQ4OTI1OCwiaGVpZ2h0IjoyLjcxODY2NjU1MzQ5NzMxNDV9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjozOC43MjUxNTg2OTE0MDYyNSwibGVmdCI6NTEuOTMyNzc3NDA0Nzg1MTU2LCJ3aWR0aCI6NDAuNjQzMTM1MDcwODAwNzgsImhlaWdodCI6MS4yNTMyODg1MDc0NjE1NDc5fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6NDAuMTE1NjEyMDMwMDI5MywibGVmdCI6NTEuOTMyNzc3NDA0Nzg1MTU2LCJ3aWR0aCI6NDAuNjY0NTg1MTEzNTI1MzksImhlaWdodCI6MS4yNTMyODg1MDc0NjE1NDc5fX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNDMxIn19XQ%3D%3D. Empirical data from generators like aQua+75MO report 168.9 nm mean size and 1.04 × 10⁹ particles/mL at 1,000 L/h, validating scalability (Nghĩa et al., 2022)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5ndXnhu4VuIEjhu691IiwiZmFtaWx5IjoiTmdoxKlhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5ndXllbiBUaGkiLCJmYW1pbHkiOiJOZ3V5ZW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUGhhbiBUcuG7jW5nIiwiZmFtaWx5IjoiQsOsbmgifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTGUgVGhpIiwiZmFtaWx5IjoiTWF5In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRvbmcgVHJhbiIsImZhbWlseSI6Ikh1eSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQaOG6oW0gVGjDoWkiLCJmYW1pbHkiOiJHaWFuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTb3BoaWUiLCJmYW1pbHkiOiJTdOKAkEhpbGFpcmUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUGhhbiBUaGkiLCJmYW1pbHkiOiJWYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMjM0LTE3NDkiLCIyMjM0LTE3NTciXSwiaXNibiI6W10sImRvaSI6IjEwLjQ3ODUzL2Zhcy4yMDIyLmUzOSIsIm9wZW5hbGV4IjoiVzQyOTQ1OTc1NTQifSwidGl0bGUiOiJFZmZlY3Qgb2YgbmFub2J1YmJsZXMgKG94eWdlbiwgb3pvbmUpIG9uIHRoZSBQYWNpZmljIHdoaXRlIHNocmltcCAoUGVuYWV1cyB2YW5uYW1laSksIFZpYnJpbyBwYXJhaGFlbW9seXRpY3VzIGFuZCB3YXRlciBxdWFsaXR5IHVuZGVyIGxhYiBjb25kaXRpb25zIiwiYWJzdHJhY3QiOiJUaGlzIHN0dWR5IGFzc2Vzc2VkIHRoZSBlZmZlY3RzIG9mIG94eWdlbiBhbmQgb3pvbmUgbmFub2J1YmJsZXMgb24gZ2lsbCBtb3JwaG9sb2d5LCB3ZWlnaHQgZ2FpbiwgYW5kIG1vcnRhbGl0eSBvZiBQYWNpZmljIHdoaXRlIHNocmltcCAoUGVuYWV1cyB2YW5uYW1laSksIGFzIHdlbGwgYXMgdGhlIGxldmVsIG9mIFZpYnJpbyBwYXJhaGFlbW9seXRpY3VzIGFuZCB3YXRlciBxdWFsaXR5IG9mIHNocmltcCBjdWx0dXJlIHRhbmtzIHVuZGVyIGxhYiBjb25kaXRpb25zLiBUd28gZXhwZXJpbWVudHMgd2VyZSBjYXJyaWVkIG91dCB3aXRoIG94eWdlbiBtYWNyb2J1YmJsZSwgb3pvbmUgbWFjcm9idWJibGUsIG94eWdlbiBuYW5vYnViYmxlLCBvem9uZSBuYW5vYnViYmxlLCBhbmQgY29udHJvbCB0cmVhdG1lbnRzIChhaXItc3RvbmUgbWFjcm9idWJibGUpLiBFeHBlcmltZW50cyB3ZXJlIGRvbmUgaW4gdHJpcGxpY2F0ZSBpbiAxMDAgTCB0YW5rcyB3aXRoIDE14oCwIHNhbGluZSB3YXRlciwgYW5kIDIwIHNocmltcCBwZXIgdGFuay4gVGFua3MgaW4gRXhwZXJpbWVudCAxIHdlcmUgbm90IGlub2N1bGF0ZWQgd2l0aCBiYWN0ZXJpYTsgdGFua3MgaW4gRXhwZXJpbWVudCAyIHdlcmUgaW5vY3VsYXRlZCB3aXRoIFYuIHBhcmFoYWVtb2x5dGljdXMgYXQgYSBjb25jZW50cmF0aW9uIG9mIDEwNiBDRlUvbUwuIFRoZSByZXN1bHRzIHJldmVhbGVkIHRoYXQgc2hvcnQgdHJlYXRtZW50cyB3aXRoIG96b25lIG5hbm9idWJibGVzIGhhZCBtaW5pbWFsIGltcGFjdCBvbiBzaHJpbXAgZ2lsbHMsIG1vcnRhbGl0eSwgYW5kIGdyb3d0aCByYXRlcywgcmVkdWNlZCBWLiBwYXJhaGFlbW9seXRpY3VzIGNvbmNlbnRyYXRpb24gaW4gd2F0ZXIgY29tcGFyZWQgdG8gdGhlIG90aGVyIGdyb3VwcywgYW5kIGltcHJvdmUgd2F0ZXIgcXVhbGl0eS4gVGhlc2UgbGFib3JhdG9yeSByZXN1bHRzIGluZGljYXRlIHRoYXQgb3pvbmUgbmFub2J1YmJsZSB0cmVhdG1lbnQgbWF5IGJlIHVzZWZ1bCBmb3IgY29udHJvbGxpbmcgVi4gcGFyYWhhZW1vbHl0aWN1cy4gTW9yZSB3b3JrIGlzIG5lZWRlZCB0byBmaW5kIHRoZSBiZXN0IHByb3RvY29sIHRvIGFwcGx5IHRoZSB0ZWNobm9sb2d5IG9uIGEgY29tbWVyY2lhbCBzY2FsZS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuNDc4NTMvZmFzLjIwMjIuZTM5IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5lLWZhcy5vcmcvZG93bmxvYWQvZG93bmxvYWRfcGRmP3BpZD1mYXMtMjUtOC00MjkiLCJpc3N1ZWREYXRlIjoiMjAyMi0wOC0wMSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZpc2hlcmllcyBhbmQgYXF1YXRpYyBzY2llbmNlcyIsImlzc3VlIjo4LCJ2b2x1bWUiOjI1LCJpbXBhY3RGYWN0b3IiOiIxLjM0In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6NDI5LCJsYXN0UGFnZSI6NDQwfX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlNwcmluZ2VyIFNjaWVuY2UrQnVzaW5lc3MgTWVkaWEifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjMwLCJjaXRlZEJ5Q291bnQiOjEzfX0sInF1b3RlIjoiQSBuYW5vYnViYmxlIGdlbmVyYXRvciBtb2RlbCBhUXVhKzc1TU8gKEFxdWFQcm8gU29sdXRpb25zLCBTaW5nYXBvcmUsIFNpbmdhcG9yZSkgd2l0aCBhIDEgSFAgcG93ZXIgKDAuNzUga1cpIGh0dHBzOi8vZG9pLm9yZy8xMC40Nzg1My9GQVMuMjAyMi5lMzkgaHR0cHM6Ly93d3cuZS1mYXMub3JnIHwgNDMxIE5ndXllbiBIdXUgTmdoaWEsIGV0IGFsLiBGaXNoZXJpZXMgYW5kIEFxdWF0aWMgU2NpZW5jZXMgbW90b3IgYW5kIGEgd2F0ZXIgZmxvdyBvdXRwdXQgcmF0ZSBvZiAxLDAwMCBML2ggd2FzIHVzZWQgdG8gZ2VuZXJhdGUgbmFub2J1YmJsZXMuIFRoZSBtYWNoaW5lIHByb2R1Y2VkIG5hbm9idWJibGVzICB3aXRoIGFuIGF2ZXJhZ2Ugc2l6ZSBvZiAxNjguOSDCsSA3My44IG5tLCBhbmQgYSBjb25jZW50cmF0aW9uIG9mICAxLjA0IMOXIDEwIDkgwrEgMi42IMOXIDEwIDggcGFydGljbGVzL21MIChBcXVhUHJvIFNvbHV0aW9ucywgMjAxOSkuIFRvIGNyZWF0ZSBveHlnZW4gbmFub2J1YmJsZXMsIHdlIHVzZWQgYW4gb3h5Z2VuIGNvbmNlbnRyYXRvciAobW9kZWwgWXV3ZWxsIDdGLTEwLCBZdXdlbGwsIERhbnlhbmcsIENoaW5hOyAxMCBML21pbiBtYXggIGZsb3dyYXRlLCDiiaUgOTUuNSUgb3h5Z2VuIGNvbmNlbnRyYXRpb24sIGZsb3cgcmF0ZSBhdCA0IEwvbWluKSAgcHJvdmlkZWQgb3h5Z2VuIGZvciBuYW5vYnViYmxlIGdlbmVyYXRvci4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo4Ni4yMjE0MDUwMjkyOTY4OCwibGVmdCI6NTQuMDQ3NTUwMjAxNDE2MDE2LCJ3aWR0aCI6MzguMjAxMjQ4MTY4OTQ1MzEsImhlaWdodCI6MS42OTk2MzEzMzMzNTExMzUzfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6ODcuOTg1Mjk4MTU2NzM4MjgsImxlZnQiOjUxLjE5MDQwNjc5OTMxNjQwNiwid2lkdGgiOjQxLjA2MDEyMzQ0MzYwMzUxNiwiaGVpZ2h0IjoxLjY5OTYzMTMzMzM1MTEzNTN9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo5NC45ODc5Mzc5MjcyNDYxLCJsZWZ0Ijo4LjA5NTIzNTgyNDU4NDk2MSwid2lkdGgiOjg0LjI4NTcyMDgyNTE5NTMxLCJoZWlnaHQiOjEuNjYxNzUwNjc0MjQ3NzQxN319LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjMuNzI4NDYxMjY1NTYzOTY1LCJsZWZ0Ijo4LjEwNDc2MjA3NzMzMTU0Mywid2lkdGgiOjE2LjI1ODc2OTk4OTAxMzY3MiwiaGVpZ2h0IjoxLjI2NjgxMDUzNjM4NDU4MjV9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo0LjcxOTI3MjEzNjY4ODIzMiwibGVmdCI6NzAuNDM0NzIyOTAwMzkwNjIsIndpZHRoIjoxNC4wMzA2OTAxOTMxNzYyNywiaGVpZ2h0IjoxLjMwMDc2OTMyOTA3MTA0NX19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjEwLjM3NDIwNDYzNTYyMDExNywibGVmdCI6OC4wOTUyMzU4MjQ1ODQ5NjEsIndpZHRoIjo0MS4wNzg3NTgyMzk3NDYwOTQsImhlaWdodCI6Ni45OTEyOTg2NzU1MzcxMDl9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjoxNS41OTYyMjc2NDU4NzQwMjMsImxlZnQiOjE0LjE2NzcxNDExODk1NzUyLCJ3aWR0aCI6Ny42NDA0NTYxOTk2NDU5OTYsImhlaWdodCI6MS43NjkyNzUxODg0NDYwNDV9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjoxNS41OTYyMjc2NDU4NzQwMjMsImxlZnQiOjIxLjc3NzMwOTQxNzcyNDYxLCJ3aWR0aCI6MjcuMzc3NTMyOTU4OTg0Mzc1LCJoZWlnaHQiOjEuNzY5Mjc1MTg4NDQ2MDQ1fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MTcuNDI5NzU4MDcxODk5NDE0LCJsZWZ0Ijo4LjA5NTIzNTgyNDU4NDk2MSwid2lkdGgiOjQxLjA2MDMwNjU0OTA3MjI2NiwiaGVpZ2h0IjoxLjY5OTYzMzEyMTQ5MDQ3ODV9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjoxOS4xOTM2NDU0NzcyOTQ5MjIsImxlZnQiOjguMDk1MjM1ODI0NTg0OTYxLCJ3aWR0aCI6NDEuMDY5ODY5OTk1MTE3MTksImhlaWdodCI6MS42OTk2MzMxMjE0OTA0Nzg1fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MjAuOTU3NTM0NzkwMDM5MDYyLCJsZWZ0Ijo4LjA5NTIzNTgyNDU4NDk2MSwid2lkdGgiOjQxLjA1ODc3Njg1NTQ2ODc1LCJoZWlnaHQiOjEuNjk5NjMzMTIxNDkwNDc4NX19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjIyLjcyMTQyNDEwMjc4MzIwMywibGVmdCI6OC4wOTUyMzU4MjQ1ODQ5NjEsIndpZHRoIjo0MC43MTQyOTA2MTg4OTY0ODQsImhlaWdodCI6MS42OTk2MzMxMjE0OTA0Nzg1fX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNDMwIn19XQ%3D%3D.

In summary, nanobubble properties-high zeta potential for stability, Laplace-driven internal pressure, and ROS generation-drive mechanisms pivotal for aquaculture water remediation, bridging nanoscale physics with macroscopic benefits. The collapse of these cavities triggers a rapid release of accumulated chemical energy within the electric double layer, while the simultaneous compression of entrapped water molecules subjects them to extreme temperatures and pressures that facilitate molecular cleavage (Wu et al., 2021)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjMwLCJzdWJ0eXBlIjp7fSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSmlhamlhIiwiZmFtaWx5IjoiV3UifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiS2VqaWEiLCJmYW1pbHkiOiJaaGFuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJDaGVuZyIsImZhbWlseSI6IkNlbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJYaWFvZ2FuZyIsImZhbWlseSI6Ild1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlJ1eWluIiwiZmFtaWx5IjoiTWFvIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Illpbmd5aW5nIiwiZmFtaWx5IjoiWmhlbmcifX1dLCJpZHMiOnsiaXNzbiI6WyIyMTkxLTA4NTUiXSwiaXNibiI6W10sImRvaSI6IjEwLjExODYvczEzNTY4LTAyMS0wMTI1NC0wIiwibWFnIjoiMzE3NTY4MzU2MSIsInBtaWQiOiIzNDE4NDEzNyIsInBtY2lkIjoiODIzOTEwOSIsIm9wZW5hbGV4IjoiVzMxNzU2ODM1NjEiLCJvcGVuQWNjZXNzSWQiOiI4NzkxMTY1NzkxMjc0ODY4MTY1In0sInRpdGxlIjoiUm9sZSBvZiBidWxrIG5hbm9idWJibGVzIGluIHJlbW92aW5nIG9yZ2FuaWMgcG9sbHV0YW50cyBpbiB3YXN0ZXdhdGVyIHRyZWF0bWVudCIsImFic3RyYWN0IjoiQWJzdHJhY3QgVGhlIG9jY3VycmVuY2Ugb2YgYSB2YXJpZXR5IG9mIG9yZ2FuaWMgcG9sbHV0YW50cyBoYXMgY29tcGxpY2F0ZWQgd2FzdGV3YXRlciB0cmVhdG1lbnQ7IHRodXMsIHRoZSBzZWFyY2ggZm9yIHN1c3RhaW5hYmxlIGFuZCBlZmZlY3RpdmUgdHJlYXRtZW50IHRlY2hub2xvZ3kgaGFzIGRyYXduIHNpZ25pZmljYW50IGF0dGVudGlvbi4gSW4gcmVjZW50IHllYXJzLCBidWxrIG5hbm9idWJibGVzLCB3aGljaCBoYXZlIGV4dHJhb3JkaW5hcnkgcHJvcGVydGllcyBkaWZmZXJpbmcgZnJvbSB0aG9zZSBvZiBtaWNyb2J1YmJsZXMsIGluY2x1ZGluZyBoaWdoIHN0YWJpbGl0eSBhbmQgbG9uZyByZXNpZGVuY2UgdGltZXMgaW4gd2F0ZXIsIGxhcmdlIHNwZWNpZmljIHN1cmZhY2UgYXJlYXMsIGhpZ2ggZ2FzIHRyYW5zZmVyIGVmZmljaWVuY3kgYW5kIGludGVyZmFjZSBwb3RlbnRpYWwsIGFuZCB0aGUgY2FwYWJpbGl0eSB0byBnZW5lcmF0ZSBmcmVlIHJhZGljYWxzLCBoYXZlIHNob3duIGF0dHJhY3RpdmUgdGVjaG5vbG9naWNhbCBhZHZhbnRhZ2VzIGFuZCBwcm9taXNpbmcgYXBwbGljYXRpb24gcHJvc3BlY3RzIGZvciB3YXN0ZXdhdGVyIHRyZWF0bWVudC4gSW4gdGhpcyByZXZpZXcsIHRoZSBiYXNpYyBjaGFyYWN0ZXJpc3RpY3Mgb2YgYnVsayBuYW5vYnViYmxlcyBhcmUgc3VtbWFyaXplZCBpbiBkZXRhaWwsIGFuZCByZWNlbnQgZmluZGluZ3MgcmVsYXRlZCB0byB0aGVpciBpbXBsZW1lbnRhdGlvbiBwYXRod2F5cyBhbmQgbWVjaGFuaXNtcyBpbiBvcmdhbmljIHdhc3Rld2F0ZXIgdHJlYXRtZW50IGFyZSBzeXN0ZW1hdGljYWxseSBkaXNjdXNzZWQsIHdoaWNoIGluY2x1ZGVzIGltcHJvdmluZyB0aGUgYWlyIGZsb3RhdGlvbiBwcm9jZXNzLCBpbmNyZWFzaW5nIHdhdGVyIGFlcmF0aW9uIHRvIHByb21vdGUgYWVyb2JpYyBiaW9sb2dpY2FsIHRlY2hub2xvZ2llcyBpbmNsdWRpbmcgYmlvbG9naWNhbCBhY3RpdmF0ZWQgY2FyYm9uLCBhY3RpdmF0ZWQgc2x1ZGdlLCBhbmQgbWVtYnJhbmUgYmlvcmVhY3RvcnMsIGFuZCBnZW5lcmF0aW5nIGFjdGl2ZSBmcmVlIHJhZGljYWxzIHRoYXQgb3hpZGlzZSBvcmdhbmljIGNvbXBvdW5kcy4gRmluYWxseSwgdGhlIGN1cnJlbnQgdGVjaG5vbG9naWNhbCBkaWZmaWN1bHRpZXMgb2YgYnVsayBuYW5vYnViYmxlcyBhcmUgYW5hbHlzZWQsIGFuZCBmdXR1cmUgZm9jdXMgYXJlYXMgZm9yIHJlc2VhcmNoIG9uIGJ1bGsgbmFub2J1YmJsZSB0ZWNobm9sb2d5IGFyZSBhbHNvIHByb3Bvc2VkLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMTg2L3MxMzU2OC0wMjEtMDEyNTQtMCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9hbWItZXhwcmVzcy5zcHJpbmdlcm9wZW4uY29tL2NvdW50ZXIvcGRmLzEwLjExODYvczEzNTY4LTAyMS0wMTI1NC0wIiwiaXNzdWVkRGF0ZSI6IjIwMjEtMDYtMjgiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTEwIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJBTUIgRXhwcmVzcyIsImlzc3VlIjoxLCJ2b2x1bWUiOjExLCJpbXBhY3RGYWN0b3IiOiI0LjI2In0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlNwcmluZ2VyIE5hdHVyZSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NzYsImNpdGVkQnlDb3VudCI6NzV9fSwicXVvdGUiOiJEdXJpbmcgdGhlIHNocmlua2FnZSBvZiBuYW5vYnViYmxlcywgdGhlIGVsZWMtdHJpYyBjaGFyZ2UgZGVuc2l0eSBhY2N1bXVsYXRlZCBpbiB0aGUgZWxlY3RyaWMgZG91YmxlICBsYXllciBpbmNyZWFzZXMgcmFwaWRseS4gQmVjYXVzZSBvZiB0aGUgZHJhc3RpYyBjaGFuZ2UgaW4gIHRoZSBnYXPigJNsaXF1aWQgaW50ZXJmYWNlIGF0IHRoZSBtb21lbnQgb2YgYnViYmxlIGNvbC1sYXBzZSwgdGhlIGhpZ2ggY29uY2VudHJhdGlvbiBvZiBpb25zIGFnZ3JlZ2F0ZWQgb24gdGhlICBpbnRlcmZhY2UgaW5zdGFudGx5IHJlbGVhc2VzIHRoZSBhY2N1bXVsYXRlZCBjaGVtaS1jYWwgZW5lcmd5LiBNZWFud2hpbGUgdGhlIGVudHJhcHBlZCB3YXRlciBtb2xlY3VsZXMgIGFyZSBzdWJqZWN0ZWQgdG8gZXh0cmVtZWx5IGhpZ2ggdGVtcGVyYXR1cmVzIGFuZCBwcmVzLXN1cmVzIHdoZW4gdGhleSBhcmUgZm9yY2VmdWxseSBjb21wcmVzc2VkIGJ5IGR5bmFtaWMgIHN0aW11bGkgKEtyaXNobmFuIGV0IGFsLiAyMDA2KSwgd2hpY2ggcmVzdWx0cyBpbiBjbGVhdmFnZSAgdGhhdCBwcm9kdWNlcyB2YXJpb3VzIHJlYWN0aXZlIG94eWdlbiBzcGVjaWVzIChST1NzKSwgIGluY2x1ZGluZyBoeWRyb3h5bCByYWRpY2FscyAowrdPSCksIHN1cGVyb3hpZGUgYW5pb24gIHJhZGljYWxzIChPMsK34oiSKSwgYW5kIHNpbmdsZXQgb3h5Z2VuICgxTzIpIChBdGtpbnNvbiBldCBhbC4gIDIwMTk7IExpdSBldCBhbC4gMjAxNTsgTHl1IGV0IGFsLiAyMDE5KS4gU2V2ZXJhbCBzdHVkaWVzIGhhdmUgIGFwcGxpZWQgZWxlY3Ryb24gc3BpbiByZXNvbmFuY2Ugb3IgdGhlIGFjdGlvbiBvZiBmcmVlIHJhZGktY2FsIGluaGliaXRvcnMgdG8gdmVyaWZ5IHRoZWlyIGV4aXN0ZW5jZS4gTWljaGFpbGlkaSBldCBhbC4gICgyMDIwKSB1c2VkIGVsZWN0cm9uIHBhcmFtYWduZXRpYyByZXNvbmFuY2UgYmFzZWQgb24gIDUsNS1kaW1ldGh5bC0xLXB5cnJvbGluZSBOLW94aWRlIChETVBPKSBzcGluIHRyYXBwaW5nICB0byBxdWFudGl0YXRpdmVseSBhbmFseXNlIGZyZWUgcmFkaWNhbHMgaW4gYWlyLW5hbm9idWJibGUgIGFuZCBveHlnZW4tbmFub2J1YmJsZSBzeXN0ZW1zLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MywiYmJveCI6eyJ0b3AiOjM1LjU0MjE2NzY2MzU3NDIyLCJsZWZ0Ijo5LjUyMzgwMDg0OTkxNDU1LCJ3aWR0aCI6MzkuMjg1NzA5MzgxMTAzNTE2LCJoZWlnaHQiOjEuODM5NDc1ODcwMTMyNDQ2M319LHsicGFnZU51bWJlciI6MywiYmJveCI6eyJ0b3AiOjM3LjA2MTMwOTgxNDQ1MzEyNSwibGVmdCI6OS41MjM4MDA4NDk5MTQ1NSwid2lkdGgiOjM5LjY3MjkxNjQxMjM1MzUxNiwiaGVpZ2h0IjoxLjgzNzY1NDU5MDYwNjY4OTV9fSx7InBhZ2VOdW1iZXIiOjMsImJib3giOnsidG9wIjozOC41NzgwMjU4MTc4NzEwOTQsImxlZnQiOjkuNTIzODAwODQ5OTE0NTUsIndpZHRoIjozOS42NzM5MTIwNDgzMzk4NDQsImhlaWdodCI6MS44Mzc2NTQ1OTA2MDY2ODk1fX0seyJwYWdlTnVtYmVyIjozLCJiYm94Ijp7InRvcCI6NDAuMDk0NzQ5NDUwNjgzNTk0LCJsZWZ0Ijo5LjUyMzgwMDg0OTkxNDU1LCJ3aWR0aCI6MzkuMjg1NzA5MzgxMTAzNTE2LCJoZWlnaHQiOjEuODM4ODY2MjMzODI1NjgzNn19LHsicGFnZU51bWJlciI6MywiYmJveCI6eyJ0b3AiOjQxLjYxMzI4NTA2NDY5NzI2NiwibGVmdCI6OS41MjM4MDA4NDk5MTQ1NSwid2lkdGgiOjM5LjY3MzQxNjEzNzY5NTMxLCJoZWlnaHQiOjEuODM3NjU0NTkwNjA2Njg5NX19LHsicGFnZU51bWJlciI6MywiYmJveCI6eyJ0b3AiOjQzLjEzMDAwMTA2ODExNTIzNCwibGVmdCI6OS41MjM4MDA4NDk5MTQ1NSwid2lkdGgiOjM5LjI4NTcwOTM4MTEwMzUxNiwiaGVpZ2h0IjoxLjgzODI2MDUzMTQyNTQ3Nn19LHsicGFnZU51bWJlciI6MywiYmJveCI6eyJ0b3AiOjQ0LjY0NzkzMzk1OTk2MDk0LCJsZWZ0Ijo5LjUyMzgwMDg0OTkxNDU1LCJ3aWR0aCI6MzkuNjczNTY4NzI1NTg1OTQsImhlaWdodCI6MS44Mzc2NTQ1OTA2MDY2ODk1fX0seyJwYWdlTnVtYmVyIjozLCJiYm94Ijp7InRvcCI6NDYuMTY0NjQ5OTYzMzc4OTA2LCJsZWZ0Ijo5LjUyMzgwMDg0OTkxNDU1LCJ3aWR0aCI6MzkuMjg1NzA5MzgxMTAzNTE2LCJoZWlnaHQiOjEuODM4MjYwNTMxNDI1NDc2fX0seyJwYWdlTnVtYmVyIjozLCJiYm94Ijp7InRvcCI6NDcuNjgyNTc5MDQwNTI3MzQ0LCJsZWZ0Ijo5LjUyMzgwMDg0OTkxNDU1LCJ3aWR0aCI6MzkuNjc3MjA0MTMyMDgwMDgsImhlaWdodCI6MS44Mzc2NTQ1OTA2MDY2ODk1fX0seyJwYWdlTnVtYmVyIjozLCJiYm94Ijp7InRvcCI6NDkuMTk5Mjk4ODU4NjQyNTgsImxlZnQiOjkuNTIzODAwODQ5OTE0NTUsIndpZHRoIjozOS42Njk0NjQxMTEzMjgxMjUsImhlaWdodCI6MS44Mzc2NTQ1OTA2MDY2ODk1fX0seyJwYWdlTnVtYmVyIjozLCJiYm94Ijp7InRvcCI6NTAuNzE2MDExMDQ3MzYzMjgsImxlZnQiOjkuNTIzODAwODQ5OTE0NTUsIndpZHRoIjozOS42NzU1NTYxODI4NjEzMywiaGVpZ2h0IjoxLjgzNzY1NDU5MDYwNjY4OTV9fSx7InBhZ2VOdW1iZXIiOjMsImJib3giOnsidG9wIjo1Mi4yMzI3MzA4NjU0Nzg1MTYsImxlZnQiOjkuNTIzODAwODQ5OTE0NTUsIndpZHRoIjozOS42NzMwODA0NDQzMzU5NCwiaGVpZ2h0IjoyLjM1OTMzODk5ODc5NDU1NTd9fSx7InBhZ2VOdW1iZXIiOjMsImJib3giOnsidG9wIjo1My42MDkzNzg4MTQ2OTcyNjYsImxlZnQiOjkuNTIzODAwODQ5OTE0NTUsIndpZHRoIjozOS42NzI4ODIwODAwNzgxMjUsImhlaWdodCI6Mi41MDE4Mzg5MjI1MDA2MTA0fX0seyJwYWdlTnVtYmVyIjozLCJiYm94Ijp7InRvcCI6NTUuMjY4NTkyODM0NDcyNjU2LCJsZWZ0Ijo5LjUyNDU5MDQ5MjI0ODUzNSwid2lkdGgiOjM5LjY2OTQ2NDExMTMyODEyNSwiaGVpZ2h0IjoxLjgzNzY1NDU5MDYwNjY4OTV9fSx7InBhZ2VOdW1iZXIiOjMsImJib3giOnsidG9wIjo1Ni43ODUzMDg4Mzc4OTA2MjUsImxlZnQiOjkuNTI0NTkwNDkyMjQ4NTM1LCJ3aWR0aCI6MzkuMjg0OTE5NzM4NzY5NTMsImhlaWdodCI6MS44Mzg4NjYyMzM4MjU2ODM2fX0seyJwYWdlTnVtYmVyIjozLCJiYm94Ijp7InRvcCI6NTguMzAzODQ0NDUxOTA0MywibGVmdCI6OS41MjM4MDA4NDk5MTQ1NSwid2lkdGgiOjM5LjY3MDk1OTQ3MjY1NjI1LCJoZWlnaHQiOjEuODM3NjU0NTkwNjA2Njg5NX19LHsicGFnZU51bWJlciI6MywiYmJveCI6eyJ0b3AiOjU5LjgyMDU2NDI3MDAxOTUzLCJsZWZ0Ijo5LjUyMzgwMDg0OTkxNDU1LCJ3aWR0aCI6MzkuNjc1MzkyMTUwODc4OTA2LCJoZWlnaHQiOjEuODM3NjU0NTkwNjA2Njg5NX19LHsicGFnZU51bWJlciI6MywiYmJveCI6eyJ0b3AiOjYxLjMzNzI4MDI3MzQzNzUsImxlZnQiOjkuNTIzODAwODQ5OTE0NTUsIndpZHRoIjozOS42NzA0NTIxMTc5MTk5MiwiaGVpZ2h0IjoxLjgzNzY1NDU5MDYwNjY4OTV9fSx7InBhZ2VOdW1iZXIiOjMsImJib3giOnsidG9wIjo2Mi44NTQwMDM5MDYyNSwibGVmdCI6OS41MjM4MDA4NDk5MTQ1NSwid2lkdGgiOjM5LjY3MDQzNjg1OTEzMDg2LCJoZWlnaHQiOjEuODM3NjU0NTkwNjA2Njg5NX19LHsicGFnZU51bWJlciI6MywiYmJveCI6eyJ0b3AiOjY0LjM3MDcxMjI4MDI3MzQ0LCJsZWZ0Ijo5LjUyMzgwMDg0OTkxNDU1LCJ3aWR0aCI6MzkuNjcwNjA4NTIwNTA3ODEsImhlaWdodCI6MS44Mzc2NTQ1OTA2MDY2ODk1fX1dfSwiY2h1bmtJZCI6IjRkNTk0YTliLTdkN2YtNDU0OS04NWEwLWFhYWU2NzdkYjk1YyIsImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjQifX1d. This phenomenon is quantitatively described by the Young-Laplace equation, P = Pₗ + 4σ/d, which dictates that internal gas pressure is substantially determined by the inverse of bubble diameter. (Hutagalung et al., 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlN1dHJpc25vIFNhbG9tbyIsImZhbWlseSI6Ikh1dGFnYWx1bmcifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW5kZSBGdWRqYSIsImZhbWlseSI6IlJhZnJ5YW50byJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJXZWkiLCJmYW1pbHkiOiJTdW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTnVyb2NobWEiLCJmYW1pbHkiOiJKdWxpYXNpaCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTcmkiLCJmYW1pbHkiOiJBZGl0aWEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSml6aG91IiwiZmFtaWx5IjoiSmlhbmcifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQXJyYW1lbCIsImZhbWlseSI6IkFycmFtZWwifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSGVybWF3YW4gS3Jlc25vIiwiZmFtaWx5IjoiRGlwb2pvbm8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU3JpIEhhcmphdGkiLCJmYW1pbHkiOiJTdWhhcmRpIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik51cnVsIFRhdWZpcXUiLCJmYW1pbHkiOiJSb2NobWFuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkRlZGR5IiwiZmFtaWx5IjoiS3VybmlhZGkifX1dLCJpZHMiOnsiaXNzbiI6WyIyMjk2LTY2NVgiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzODkvZmVudnMuMjAyMy4xMTU0NzM5Iiwib3BlbmFsZXgiOiJXNDMyODExNDkyMyIsIm9wZW5BY2Nlc3NJZCI6IjkwMTA5ODQ0NTE0ODE2MTM4NzQifSwidGl0bGUiOiJDb21iaW5hdGlvbiBvZiBvem9uZS1iYXNlZCBhZHZhbmNlZCBveGlkYXRpb24gcHJvY2VzcyBhbmQgbmFub2J1YmJsZXMgZ2VuZXJhdGlvbiB0b3dhcmQgdGV4dGlsZSB3YXN0ZXdhdGVyIHJlY292ZXJ5IiwiYWJzdHJhY3QiOiJUaGUgaW50cmljYXRlIG5hdHVyZSBvZiB2YXJpb3VzIHRleHRpbGUgbWFudWZhY3R1cmluZyBwcm9jZXNzZXMgaW50cm9kdWNlcyBjb2xvcmVkIGR5ZXMsIHN1cmZhY3RhbnRzLCBhbmQgdG94aWMgY2hlbWljYWxzIHRoYXQgaGF2ZSBiZWVuIGhhcm1mdWwgdG8gZWNvc3lzdGVtcyBpbiByZWNlbnQgeWVhcnMuIEhlcmUsIGEgY29tYmluYXRpb24gb3pvbmUtYmFzZWQgYWR2YW5jZWQgb3hpZGF0aW9uIHByb2Nlc3MgKEFPUCkgaXMgY291cGxlZCB3aXRoIGEgbmFub2J1YmJsZXMgZ2VuZXJhdG9yIGZvciB0aGUgZ2VuZXJhdGlvbiBvZiBvem9uZSBuYW5vYnViYmxlcyAoTkIpIHV0aWxpemVkIHRoZSBzYW1lIHRvIHRyZWF0IHRoZSBwcmltYXJ5IGVmZmx1ZW50IGFjcXVpcmVkIGZyb20gdGV4dGlsZSB3YXN0ZXdhdGVycy4gSGVyZSB3ZSBmaW5kIHNldmVyYWwga2V5IHBhcmFtZXRlcnMgc3VjaCBhcyBjaGVtaWNhbCBveHlnZW4gZGVtYW5kIGFtbW9uaWEgY29udGVudCAoTkggMyApLCBhbmQgdG90YWwgc3VzcGVuZGVkIHNvbGlkcyBpbmRpY2F0aW5nIGEgc3Vic3RhbnRpYWwgcmVjb3ZlcnkgaW4gd2hpY2ggdGhlIHJlc3BlY3RpdmUgcGVyY2VudGFnZXMgb2YgODEuMSUsIDMwLjgxJSwgYW5kIDQxLjk4JSwgdXBvbiAzMDAgbWluIHJlc2lkZW5jZSB0aW1lIGFyZSBhY2hpZXZlZC4gT24gdGhlIG90aGVyIGhhbmQsIHRoZSBwSCBpcyBzaGlmdGVkIGZyb20gNy45MyB0byA3LjQ2LCBpbmRpY2F0aW5nIHRoZSBnZW5lcmF0aW9uIG9mIGh5ZHJvZ2VuIHBlcm94aWRlIChIIDIgTyAyICkgZHVlIHRvIHRoZSB0ZXJtaW5hdGlvbiByZWFjdGlvbiBhbmQgdGhlIHNlbGYtcmVhY3Rpb24gb2YgcmVhY3RpdmUgb3h5Z2VuIHNwZWNpZXMgKFJPUykuIFdlIHByb3Bvc2UgdGhhdCB0aGUgcmVhY3RpdmUgb3h5Z2VuIHNwZWNpZXMgY2FuIGJlIGlkZW50aWZpZWQgZnJvbSB0aGUgbmVnYXRpdmUgemV0YSBwb3RlbnRpYWwgbWVhc3VyZW1lbnQgKOKIkjIyLjQzIMKxIDAuMzQgbVYpIGNvbGxlY3RlZCBpbiB0aGUgZmluYWwgc3RhdGUgb2YgdHJlYXRtZW50LiBUaGUgY29tYmluZWQgbWV0aG9kIGhhcyBzdWNjZXNzZnVsbHkgZ2VuZXJhdGVkIG96b25lIG5hbm9idWJibGVzIHdpdGggOTkuOTQlIG9mIHNpemUgZGlzdHJpYnV0ZWQgaW4gMjE2Ljkgbm0uIFRoaXMgaGlnaGxpZ2h0cyB0aGF0IGVuaGFuY2VtZW50IG9mIG96b25l4oCZcyByZWFjdGl2aXR5IHBsYXlzIGEgY3J1Y2lhbCByb2xlIGluIGltcHJvdmluZyB0aGUgd2F0ZXIgcXVhbGl0eSBvZiB0ZXh0aWxlIHdhc3Rld2F0ZXIgdG93YXJkcyBiZWluZyB0ZWNobm9sb2dpY2FsbHkgZWZmaWNpZW50IHRvIGRhdGUuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjMzODkvZmVudnMuMjAyMy4xMTU0NzM5IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5mcm9udGllcnNpbi5vcmcvYXJ0aWNsZXMvMTAuMzM4OS9mZW52cy4yMDIzLjExNTQ3MzkvcGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjMtMDMtMjEiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTEwIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJGcm9udGllcnMgaW4gRW52aXJvbm1lbnRhbCBTY2llbmNlIiwidm9sdW1lIjoxMSwiaW1wYWN0RmFjdG9yIjoiNS42NCJ9LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJGcm9udGllcnMgTWVkaWEifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjY0LCJjaXRlZEJ5Q291bnQiOjE5fX0sInF1b3RlIjoiT3VyIGZpbmRpbmcgcmVnYXJkaW5nIENPRCByZWR1Y3Rpb24gdGhhdCBvcGVyYXRlZCBhdCBuZXV0cmFsIHBIIGluIGFncmVlbWVudCB3aXRoIHRoZSBwcmV2aW91cyByZXBvcnRzIChMdWNhcyBldCBhbC4sIDIwMDk7IFRha2FoYXNoaSBldCBhbC4sIDIwMTI7IFlhbmcgZXQgYWwuLCAyMDEyOyBUZW1lc2dlbiBldCBhbC4sIDIwMTcpLiBUaGUgc2Vjb25kIHJlY292ZXJ5IHN0YWdlIGNvcnJlc3BvbmRzIHRvIHRoZSB1dGlsaXphdGlvbiBvZiBvem9uZS1iYXNlZCBBT1AgYXMgaW5kaWNhdGVkIGJ5IHRoZSBibHVlIGFycm93IGluIEZpZ3VyZSA3LiBJbiB0aGlzIHNlY3Rpb24sIHRoZSBnZW5lcmF0aW9uIG9mIFJPUyBvciBuZWdhdGl2ZWx5LWNoYXJnZWQgTkIgZG9taW5hbnRseSBjb250cmlidXRlZCB0byBkZWdyYWRlIHRoZSBvcmdhbmljIGNvbnRhbWluYW50cy4gV2UgcmV2aXNpdCB0aGUgY29udmVudGlvbmFsIHRoZXJtb2R5bmFtaWNzIG9mIGJ1YmJsZSBjb2xsYXBzZSBwaGVub21lbm9uIHRvIGRlc2NyaWJlIHRoZSBST1MgZ2VuZXJhdGlvbiAoR2hhZGlta2hhbmkgZXQgYWwuLCAyMDE2KSBhbmQgb3pvbmUgYnJlYWtkb3duIChJa2hsYXEgZXQgYWwuLCAyMDEzOyBPaCBhbmQgTmd1eWVuLCAyMDIyKS4gSW4gdGhlIG96b25lIGRlY29tcG9zaXRpb24sIHdlIGF0dHJpYnV0ZSB0aGUgUk9TIGlzIGEgY2hhaW4gcmVhY3Rpb24gY29tcHJpc2VkIG9mIGluaXRpYXRpb24sIHByb3BhZ2F0aW9uLCBhbmQgdGVybWluYXRpb24gKEtodW50aWEgZXQgYWwuLCAyMDE1KS4gRnVydGhlcm1vcmUsIFJPUyB2aWEgY29sbGFwc2luZyBjYXZpdHkgYnViYmxlcyBjYW4gYmUgZGVzY3JpYmVkIGJhc2VkIG9uIHRoZSBZb3VuZy1MYXBsYWNlIGVxdWF0aW9uIChLaW11cmEgZXQgYWwuLCAyMDA0OyBUYWthaGFzaGkgZXQgYWwuLCAyMDA3OyBHaGFkaW1raGFuaSBldCBhbC4sIDIwMTYpLiBQIO%2B%2FvSBQMSArIDTPg%2B%2B%2FvWQgKDMpIHdoZXJlIFAgZGVub3RlcyBnYXMgcHJlc3N1cmUsIFBsIGRlbm90ZXMgbGlxdWlkIHByZXNzdXJlLCDPgyBkZW5vdGVzIGxpcXVpZCBzdXJmYWNlIHRlbnNpb24sIGFuZCBkIGRlbm90ZXMgdGhlIGJ1YmJsZSBkaWFtZXRlci4gVGhlIGludGVybmFsIHByZXNzdXJlIG9mIGEgYnViYmxlIGlzIHN1YnN0YW50aWFsbHkgZGV0ZXJtaW5lZCBieSBpdHMgc2l6ZSwgYWNjb3JkaW5nIHRvIEVxLiAzLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjU5LjI3ODg5MjUxNzA4OTg0NCwibGVmdCI6OC4zMzMxNzI3OTgxNTY3MzgsIndpZHRoIjo0MC40NzE4MzYwOTAwODc4OSwiaGVpZ2h0IjoxLjE3NTc4MzUxNDk3NjUwMTV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo2MC42OTk3Mzc1NDg4MjgxMjUsImxlZnQiOjguMzMzMTcwODkwODA4MTA1LCJ3aWR0aCI6NDAuNDcwOTMyMDA2ODM1OTQsImhlaWdodCI6MS4xNzU3ODM1MTQ5NzY1MDE1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NjIuMTIwNTc0OTUxMTcxODc1LCJsZWZ0Ijo4LjMzMzE3MDg5MDgwODEwNSwid2lkdGgiOjQwLjQ3MTM4MjE0MTExMzI4LCJoZWlnaHQiOjEuMTc1NzgzNTE0OTc2NTAxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjYzLjUzNDcxNzU1OTgxNDQ1LCJsZWZ0Ijo4LjMzMzE3Mjc5ODE1NjczOCwid2lkdGgiOjE0LjEwMjE5NjY5MzQyMDQxLCJoZWlnaHQiOjEuMTc1NzgzNTE0OTc2NTAxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjY0Ljk1NTU1ODc3Njg1NTQ3LCJsZWZ0IjoxMC42NzU5ODQzODI2MjkzOTUsIndpZHRoIjozOC4xMzI5NDIxOTk3MDcwMywiaGVpZ2h0IjoxLjE3NTc4MzUxNDk3NjUwMTV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo2Ni4zNzY0MDM4MDg1OTM3NSwibGVmdCI6OC4zMzMxNzU2NTkxNzk2ODgsIndpZHRoIjo0MC40NzA5MzIwMDY4MzU5NCwiaGVpZ2h0IjoxLjE3NTc4MzUxNDk3NjUwMTV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo2Ny43OTcyNDg4NDAzMzIwMywibGVmdCI6OC4zMzMxNzc1NjY1MjgzMiwid2lkdGgiOjQuNTQ0MzMyOTgxMTA5NjE5LCJoZWlnaHQiOjEuMTc1NzgzNTE0OTc2NTAxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjY3Ljc5NzI0ODg0MDMzMjAzLCJsZWZ0IjoxNC4xOTAyMDE3NTkzMzgzNzksIndpZHRoIjoxLjg4MTI5NzcwNzU1NzY3ODIsImhlaWdodCI6MS4xNzU3ODM1MTQ5NzY1MDE1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NjcuNzk3MjQ4ODQwMzMyMDMsImxlZnQiOjE3LjM3MTA4NjEyMDYwNTQ3LCJ3aWR0aCI6Ni4zNzg5MzgxOTgwODk2LCJoZWlnaHQiOjEuMTc1NzgzNTE0OTc2NTAxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjY3Ljc5NzI0ODg0MDMzMjAzLCJsZWZ0IjoyNS4wNjYwMjY2ODc2MjIwNywid2lkdGgiOjEuMTkxNDM4MzE3Mjk4ODg5MiwiaGVpZ2h0IjoxLjE3NTc4MzUxNDk3NjUwMTV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo2Ny43OTcyNDg4NDAzMzIwMywibGVmdCI6MjcuNTYxMjcxNjY3NDgwNDcsIndpZHRoIjoyLjc0Mjg2ODE4NTA0MzMzNSwiaGVpZ2h0IjoxLjE3NTc4MzUxNDk3NjUwMTV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo2Ny43OTcyNDg4NDAzMzIwMywibGVmdCI6MzEuNjA4ODQ2NjY0NDI4NzEsIndpZHRoIjoxLjMwMTM5NDIyNDE2Njg3MDEsImhlaWdodCI6MS4xNzU3ODM1MTQ5NzY1MDE1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NjcuNzk3MjQ4ODQwMzMyMDMsImxlZnQiOjM0LjIxODQxNDMwNjY0MDYyNSwid2lkdGgiOjExLjI4NDc3MTkxOTI1MDQ4OCwiaGVpZ2h0IjoxLjE3NTc4MzUxNDk3NjUwMTV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo2Ny43OTcyNDg4NDAzMzIwMywibGVmdCI6NDYuODE3OTg1NTM0NjY3OTcsIndpZHRoIjoxLjk5MTI1MzQ5NTIxNjM2OTYsImhlaWdodCI6MS4xNzU3ODM1MTQ5NzY1MDE1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NjkuMjE4MDkzODcyMDcwMzEsImxlZnQiOjguMzMzMTc3NTY2NTI4MzIsIndpZHRoIjo0MC40NzM3OTMwMjk3ODUxNTYsImhlaWdodCI6MS4xNzU3ODM1MTQ5NzY1MDE1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NzAuNjM4OTM4OTAzODA4NiwibGVmdCI6OC4zMzMxNzc1NjY1MjgzMiwid2lkdGgiOjQwLjQ3MTk4ODY3Nzk3ODUxNiwiaGVpZ2h0IjoxLjE3NTc4MzUxNDk3NjUwMTV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo3Mi4wNTk3NzYzMDYxNTIzNCwibGVmdCI6OC4zMzMxNzc1NjY1MjgzMiwid2lkdGgiOjQwLjQ2NjQxMTU5MDU3NjE3LCJoZWlnaHQiOjEuMTc1NzgzNTE0OTc2NTAxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjczLjQ4MDYyMTMzNzg5MDYyLCJsZWZ0Ijo4LjMzMzE3NzU2NjUyODMyLCJ3aWR0aCI6NDAuNDc1NTk3MzgxNTkxOCwiaGVpZ2h0IjoxLjE3NTc4MzUxNDk3NjUwMTV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo3NC44OTQ3NTI1MDI0NDE0LCJsZWZ0Ijo4LjMzMzE3NzU2NjUyODMyLCJ3aWR0aCI6NDAuNDcyMjkzODUzNzU5NzY2LCJoZWlnaHQiOjEuMTc1NzgzNTE0OTc2NTAxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjc2LjMxNTU5NzUzNDE3OTY5LCJsZWZ0Ijo4LjMzMzE3ODUyMDIwMjYzNywid2lkdGgiOjAuNjU4MjI4MDk5MzQ2MTYwOSwiaGVpZ2h0IjoxLjE3NTc4MzUxNDk3NjUwMTV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo3Ni4zMTU1OTc1MzQxNzk2OSwibGVmdCI6MTAuMTk5ODY1MzQxMTg2NTIzLCJ3aWR0aCI6MTYuODU1MTU1OTQ0ODI0MjIsImhlaWdodCI6MS4xNzU3ODM1MTQ5NzY1MDE1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NzYuMzE1NTk3NTM0MTc5NjksImxlZnQiOjI4LjI2NjAzODg5NDY1MzMyLCJ3aWR0aCI6MS4xOTE0MzgzMTcyOTg4ODkyLCJoZWlnaHQiOjEuMTc1NzgzNTE0OTc2NTAxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjc2LjMxNTU5NzUzNDE3OTY5LCJsZWZ0IjozMC42NjU5Mzc0MjM3MDYwNTUsIndpZHRoIjo1Ljg0NTcyNzkyMDUzMjIyNywiaGVpZ2h0IjoxLjE3NTc4MzUxNDk3NjUwMTV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo3Ni4zMTU1OTc1MzQxNzk2OSwibGVmdCI6MzcuNzIyODM5MzU1NDY4NzUsIndpZHRoIjo3LjYxNzA2OTcyMTIyMTkyNCwiaGVpZ2h0IjoxLjE3NTc4MzUxNDk3NjUwMTV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo3Ni4zMTU1OTc1MzQxNzk2OSwibGVmdCI6NDYuNTUxMDc4Nzk2Mzg2NzIsIndpZHRoIjoyLjI1NjM1MTIzMjUyODY4NjUsImhlaWdodCI6MS4xNzU3ODM1MTQ5NzY1MDE1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NzcuNzM2NDUwMTk1MzEyNSwibGVmdCI6OC4zMzMxNzg1MjAyMDI2MzcsIndpZHRoIjoyMS4wODMwMjY4ODU5ODYzMjgsImhlaWdodCI6MS4xNzU3ODM1MTQ5NzY1MDE1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NzkuMTU3Mjk1MjI3MDUwNzgsImxlZnQiOjEwLjY3NTk4NzI0MzY1MjM0NCwid2lkdGgiOjM4LjEyOTkyMDk1OTQ3MjY1NiwiaGVpZ2h0IjoxLjE3NTc4MzUxNDk3NjUwMTV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo4MC41NzgxNDAyNTg3ODkwNiwibGVmdCI6OC4zMzMwMjc4Mzk2NjA2NDUsIndpZHRoIjo0MC40Nzc4NTU2ODIzNzMwNSwiaGVpZ2h0IjoxLjE3NTc4MzUxNDk3NjUwMTV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo4MS45OTg5Nzc2NjExMzI4MSwibGVmdCI6OC4zMzMwMjY4ODU5ODYzMjgsIndpZHRoIjozOC4xNDc1NDQ4NjA4Mzk4NDQsImhlaWdodCI6MS4xNzU3ODM1MTQ5NzY1MDE1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6ODMuMTAxMzQxMjQ3NTU4NiwibGVmdCI6MjQuMjA5MjcwNDc3Mjk0OTIyLCJ3aWR0aCI6MjQuNjAyNTkyNDY4MjYxNzIsImhlaWdodCI6NS4zNDY0MDY0NTk4MDgzNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjg2LjE4MTM4MTIyNTU4NTk0LCJsZWZ0Ijo4LjMzMzI1NDgxNDE0Nzk1LCJ3aWR0aCI6NDAuNDc0NDE0ODI1NDM5NDUsImhlaWdodCI6MS4zNTc2MDc5NjA3MDA5ODg4fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6ODcuNjAyMjI2MjU3MzI0MjIsImxlZnQiOjguMzMzMjc3NzAyMzMxNTQzLCJ3aWR0aCI6NDAuNDc0NTQ0NTI1MTQ2NDg0LCJoZWlnaHQiOjEuMTc1NzgzNTE0OTc2NTAxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjg5LjAyMzA3ODkxODQ1NzAzLCJsZWZ0Ijo4LjMzMzQzMDI5MDIyMjE2OCwid2lkdGgiOjQwLjQ3Mjc0MDE3MzMzOTg0NCwiaGVpZ2h0IjoxLjE3NTc4MzUxNDk3NjUwMTV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo5MC40NDM5MjM5NTAxOTUzMSwibGVmdCI6OC4zMzM0MzAyOTAyMjIxNjgsIndpZHRoIjo0MC40NzY4MDY2NDA2MjUsImhlaWdodCI6MS4xNzU3ODM1MTQ5NzY1MDE1fX1dfSwiY2h1bmtJZCI6IjExYTM5MDc4LTFhYmYtNDg2Yy05OTUyLWIyZTljNGNhYzk3NyIsImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjYifX1d.

2.3. Applications in Aquaculture

2.3.1. Water Quality Improvement

Nanobubbles represent a promising technology for enhancing water quality parameters in aquaculture systems, addressing key challenges such as low dissolved oxygen levels and accumulation of toxic nitrogenous wastes (Nghĩa et al., 2022)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5ndXnhu4VuIEjhu691IiwiZmFtaWx5IjoiTmdoxKlhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5ndXllbiBUaGkiLCJmYW1pbHkiOiJOZ3V5ZW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUGhhbiBUcuG7jW5nIiwiZmFtaWx5IjoiQsOsbmgifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTGUgVGhpIiwiZmFtaWx5IjoiTWF5In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRvbmcgVHJhbiIsImZhbWlseSI6Ikh1eSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQaOG6oW0gVGjDoWkiLCJmYW1pbHkiOiJHaWFuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTb3BoaWUiLCJmYW1pbHkiOiJTdOKAkEhpbGFpcmUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUGhhbiBUaGkiLCJmYW1pbHkiOiJWYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMjM0LTE3NDkiLCIyMjM0LTE3NTciXSwiaXNibiI6W10sImRvaSI6IjEwLjQ3ODUzL2Zhcy4yMDIyLmUzOSIsIm9wZW5hbGV4IjoiVzQyOTQ1OTc1NTQifSwidGl0bGUiOiJFZmZlY3Qgb2YgbmFub2J1YmJsZXMgKG94eWdlbiwgb3pvbmUpIG9uIHRoZSBQYWNpZmljIHdoaXRlIHNocmltcCAoUGVuYWV1cyB2YW5uYW1laSksIFZpYnJpbyBwYXJhaGFlbW9seXRpY3VzIGFuZCB3YXRlciBxdWFsaXR5IHVuZGVyIGxhYiBjb25kaXRpb25zIiwiYWJzdHJhY3QiOiJUaGlzIHN0dWR5IGFzc2Vzc2VkIHRoZSBlZmZlY3RzIG9mIG94eWdlbiBhbmQgb3pvbmUgbmFub2J1YmJsZXMgb24gZ2lsbCBtb3JwaG9sb2d5LCB3ZWlnaHQgZ2FpbiwgYW5kIG1vcnRhbGl0eSBvZiBQYWNpZmljIHdoaXRlIHNocmltcCAoUGVuYWV1cyB2YW5uYW1laSksIGFzIHdlbGwgYXMgdGhlIGxldmVsIG9mIFZpYnJpbyBwYXJhaGFlbW9seXRpY3VzIGFuZCB3YXRlciBxdWFsaXR5IG9mIHNocmltcCBjdWx0dXJlIHRhbmtzIHVuZGVyIGxhYiBjb25kaXRpb25zLiBUd28gZXhwZXJpbWVudHMgd2VyZSBjYXJyaWVkIG91dCB3aXRoIG94eWdlbiBtYWNyb2J1YmJsZSwgb3pvbmUgbWFjcm9idWJibGUsIG94eWdlbiBuYW5vYnViYmxlLCBvem9uZSBuYW5vYnViYmxlLCBhbmQgY29udHJvbCB0cmVhdG1lbnRzIChhaXItc3RvbmUgbWFjcm9idWJibGUpLiBFeHBlcmltZW50cyB3ZXJlIGRvbmUgaW4gdHJpcGxpY2F0ZSBpbiAxMDAgTCB0YW5rcyB3aXRoIDE14oCwIHNhbGluZSB3YXRlciwgYW5kIDIwIHNocmltcCBwZXIgdGFuay4gVGFua3MgaW4gRXhwZXJpbWVudCAxIHdlcmUgbm90IGlub2N1bGF0ZWQgd2l0aCBiYWN0ZXJpYTsgdGFua3MgaW4gRXhwZXJpbWVudCAyIHdlcmUgaW5vY3VsYXRlZCB3aXRoIFYuIHBhcmFoYWVtb2x5dGljdXMgYXQgYSBjb25jZW50cmF0aW9uIG9mIDEwNiBDRlUvbUwuIFRoZSByZXN1bHRzIHJldmVhbGVkIHRoYXQgc2hvcnQgdHJlYXRtZW50cyB3aXRoIG96b25lIG5hbm9idWJibGVzIGhhZCBtaW5pbWFsIGltcGFjdCBvbiBzaHJpbXAgZ2lsbHMsIG1vcnRhbGl0eSwgYW5kIGdyb3d0aCByYXRlcywgcmVkdWNlZCBWLiBwYXJhaGFlbW9seXRpY3VzIGNvbmNlbnRyYXRpb24gaW4gd2F0ZXIgY29tcGFyZWQgdG8gdGhlIG90aGVyIGdyb3VwcywgYW5kIGltcHJvdmUgd2F0ZXIgcXVhbGl0eS4gVGhlc2UgbGFib3JhdG9yeSByZXN1bHRzIGluZGljYXRlIHRoYXQgb3pvbmUgbmFub2J1YmJsZSB0cmVhdG1lbnQgbWF5IGJlIHVzZWZ1bCBmb3IgY29udHJvbGxpbmcgVi4gcGFyYWhhZW1vbHl0aWN1cy4gTW9yZSB3b3JrIGlzIG5lZWRlZCB0byBmaW5kIHRoZSBiZXN0IHByb3RvY29sIHRvIGFwcGx5IHRoZSB0ZWNobm9sb2d5IG9uIGEgY29tbWVyY2lhbCBzY2FsZS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuNDc4NTMvZmFzLjIwMjIuZTM5IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5lLWZhcy5vcmcvZG93bmxvYWQvZG93bmxvYWRfcGRmP3BpZD1mYXMtMjUtOC00MjkiLCJpc3N1ZWREYXRlIjoiMjAyMi0wOC0wMSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZpc2hlcmllcyBhbmQgYXF1YXRpYyBzY2llbmNlcyIsImlzc3VlIjo4LCJ2b2x1bWUiOjI1LCJpbXBhY3RGYWN0b3IiOiIxLjM0In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6NDI5LCJsYXN0UGFnZSI6NDQwfX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlNwcmluZ2VyIFNjaWVuY2UrQnVzaW5lc3MgTWVkaWEifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjMwLCJjaXRlZEJ5Q291bnQiOjEzfX0sInF1b3RlIjoiVGhlIGluZHVzdHJ5IHRoZXJlZm9yZSBuZWVkcyB0byBmaW5kIGFsdGVybmF0aXZlIG1lYXN1cmVzICB0byBwcmV2ZW50IGFuZCBjb250cm9sIHRoaXMgcGF0aG9nZW4uIE5hbm9idWJibGUgdGVjaG5vbG9neSBwcm9kdWNlcyBzbWFsbCBnYXMgYnViYmxlcyB3aXRoIGEgZGlhbWV0ZXIgb2YgbGVzcyB0aGFuIDIwMCBubSAoQWdyYXdhbCBldCBhbC4sIDIwMTE7IENoYXBsaW4sIDIwMjEpIHdoaWNoIGNhbiBleGlzdCBpbiB0aGUgd2F0ZXIgY29sdW1uIGZvciB3ZWVrcyAoQXpldmVkbyAgZXQgYWwuLCAyMDE2OyBQYXJtYXIgJiBNYWp1bWRlciwgMjAxMykuIFRoaXMgcGhlbm9tZW5vbiBpcyAgYXNzb2NpYXRlZCB3aXRoIHRoZSBwaHlzaWNhbCBjaGFyYWN0ZXJpc3RpYyBvZiB0aGVzZSBzbWFsbCBidWItYmxlcyAoV2VpanMgJiBMb2hzZSwgMjAxMykgYW5kIGhpZ2ggemV0YSBwb3RlbnRpYWwgdmFsdWUgd2hpY2ggIGtlZXBzIHRoZW0gZnJvbSBhZ2dyZWdhdGluZyBpbnRvIGxhcmdlciBidWJibGVzIChLaXJieSwgMjAxMDsgIFNqb2dyZWVuIGV0IGFsLiwgMjAxOCkuIE5hbm9idWJibGUgdGVjaG5vbG9neSBoYXMgYSBsb3Qgb2YgcHJvbS1pc2luZyBlZmZlY3RpdmUgYXBwbGljYXRpb24gaW4gYXF1YWN1bHR1cmUgYmVjYXVzZSBvZiBpdHMgYWJpbGl0eSAgdG8gaW5jcmVhc2UgYW5kIG1haW50YWluIG94eWdlbiBjb250ZW50IGluIHdhdGVyIGFuZCBjb250cm9sICBiYWN0ZXJpYSwgYSBwb3NzaWJsZSBtZWFzdXJlIHRvIHJlZHVjZSB0aGUgdXNlIG9mIGFudGliaW90aWNzLiBPeHlnZW4gbmFub2J1YmJsZXMgb3B0aW1pemUgdGhlIGRpc3NvbHZlZCBveHlnZW4gaW4gYXF1YXRpYyBzeXN0ZW1zIChHYWxhbmcgZXQgYWwuLCAyMDE5KS4gSGlnaCBsZXZlbHMgb2Ygb3h5Z2VuIG1heSBzcGVlZCB1cCBtZXRhYm9saXNtLCBwcm9tb3RlIGhpZ2hlciBmb29kIGludGFrZSwgYW5kIGluY3JlYXNlICBncm93dGggb2YgZmFybWVkIHNocmltcCAoRWJpbmEgZXQgYWwuLCAyMDEzKS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNS4wNjg2NDU0NzcyOTQ5MiwibGVmdCI6OC4wOTUyMzU4MjQ1ODQ5NjEsIndpZHRoIjo0MS4wNzIyMTIyMTkyMzgyOCwiaGVpZ2h0IjoxLjY5OTYzNTE0ODA0ODQwMDl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNi44MzI1MzA5NzUzNDE4LCJsZWZ0Ijo4LjA5NTIzNTgyNDU4NDk2MSwid2lkdGgiOjIzLjYwNDE4NTEwNDM3MDExNywiaGVpZ2h0IjoxLjY5OTYzNTE0ODA0ODQwMDl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC41OTY0MjAyODgwODU5NCwibGVmdCI6MTAuOTUyMzY0OTIxNTY5ODI0LCJ3aWR0aCI6MzguMjE1ODc3NTMyOTU4OTg0LCJoZWlnaHQiOjEuNjk5NjM1MTQ4MDQ4NDAwOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQwLjM2MDMwOTYwMDgzMDA4LCJsZWZ0Ijo4LjA5NTIzNTgyNDU4NDk2MSwid2lkdGgiOjQxLjA3Njc1OTMzODM3ODkwNiwiaGVpZ2h0IjoxNy41NzQ2MzA3MzczMDQ2ODh9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1Ny45OTkxOTUwOTg4NzY5NSwibGVmdCI6MTAuOTUyMzY0OTIxNTY5ODI0LCJ3aWR0aCI6MzguMjAxNDQ2NTMzMjAzMTI1LCJoZWlnaHQiOjEuNjk5NjM1MTQ4MDQ4NDAwOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjU5Ljc2MzA4MDU5NjkyMzgzLCJsZWZ0Ijo4LjA5NTIzNTgyNDU4NDk2MSwid2lkdGgiOjQxLjA2MzMyNzc4OTMwNjY0LCJoZWlnaHQiOjEuNjk5NjM1MTQ4MDQ4NDAwOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjYxLjUyNjk3MzcyNDM2NTIzNCwibGVmdCI6OC4wOTUyMzU4MjQ1ODQ5NjEsIndpZHRoIjo0MS4wNjY4NDQ5NDAxODU1NSwiaGVpZ2h0IjoxLjY5OTYzNTE0ODA0ODQwMDl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2My4yOTA4NjMwMzcxMDkzNzUsImxlZnQiOjguMDk1MjM1ODI0NTg0OTYxLCJ3aWR0aCI6NDAuNzE0MjkwNjE4ODk2NDg0LCJoZWlnaHQiOjEuNjk5NjMxMzMzMzUxMTM1M319XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjQzMCJ9fV0%3D. In intensive farming environments, where high stocking densities often lead to hypoxia and elevated ammonia concentrations, nanobubbles offer superior oxygen transfer efficiency compared to traditional aeration methods. Their nanoscale size (typically <1 μm) results in a high surface area-to-volume ratio, Brownian motion, and low buoyancy, enabling prolonged residence time and supersaturation of DO up to 200-300% (Tekile et al., 2016)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFuZGluZXQgS2ViZWRlIiwiZmFtaWx5IjoiVGVraWxlIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikls4oCQSG8iLCJmYW1pbHkiOiJLaW0ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSmFpeWVvcCIsImZhbWlseSI6IkxlZSJ9fV0sImlkcyI6eyJpc3NuIjpbIjEyMjYtMTAyNSIsIjIwMDUtOTY4WCJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuNDQ5MS9lZXIuMjAxNi4wMjgiLCJtYWciOiIyNTIzNjY5NjU3Iiwib3BlbmFsZXgiOiJXMjUyMzY2OTY1NyJ9LCJ0aXRsZSI6IkV4dGVudCBhbmQgcGVyc2lzdGVuY2Ugb2YgZGlzc29sdmVkIG94eWdlbiBlbmhhbmNlbWVudCB1c2luZyBuYW5vYnViYmxlcyIsImFic3RyYWN0IjoiSW4gdGhpcyBzdHVkeSwgY2hhbmdlIGluIHdhdGVyLWRpc3NvbHZlZCBveHlnZW4gKERPKSB3YXMgYW5hbHl6ZWQgdW5kZXIgdmFyaW91cyBzeW50aGV0aWMgd2F0ZXIgcXVhbGl0aWVzIGFuZCBuYW5vYnViYmxlcyAoTkJzKSBhcHBsaWNhdGlvbiBjb25kaXRpb25zLCBzdWNoIGFzIGdhcyB0eXBlLCBpbml0aWFsIERPIGFzIHdlbGwgYXMgd2F0ZXIgZGlzc29sdmVkLCBzdXNwZW5kZWQgYW5kIG9yZ2FuaWMgbWF0dGVycyBjb250ZW50cy4gV2hlbiBveHlnZW4sIHJhdGhlciB0aGFuIGFpciwgd2FzIGludHJvZHVjZWQgaW50byBuaXRyb2dlbi1kZXNvcmJlZCB1bHRyYS1wdXJlIHdhdGVyLCB0aGUgc3RhZ25hdGlvbiB0aW1lIHdhcyBzaWduaWZpY2FudGx5IGluY3JlYXNlZC4gSXQgdG9vayB0ZW4gZGF5cyBmb3IgRE8gY29uY2VudHJhdGlvbiB0byBkcm9wIGJhY2sgdG8gc2F0dXJhdGlvbi4gVGhlIGhpZ2hlciB0aGUgaW5pdGlhbCBETyBjb25jZW50cmF0aW9uLCB0aGUgbG9uZ2VyIHBhcnRpY2xlcyB3ZXJlIG9ic2VydmVkIGFib3ZlIHNhdHVyYXRpb24gZHVlIHRvIHBhcnRpY2xlIHN0YWJpbGl0eSBpbXByb3ZlbWVudC4gVGhlIG94eWdlbiBtYXNzIHRyYW5zZmVyIHJhdGUgb2YgMC4wNDgyIG1nL0wvbWluIHdhcyBmb3VuZCB0byByZWFjaCBhIG1heGltdW0gYXQgYW4gZWxlY3Ryb2x5dGljIGNvbmNlbnRyYXRpb24gb2YgMC43NSBnL0wsIGJleW9uZCB3aGljaCB0aGUgdHJhbnNmZXIgcmF0ZSBkZWNyZWFzZWQgZHVlIHRvIGFkc29ycHRpb24gb2YgbmVnYXRpdmUgaW9ucyBvZiB0aGUgZWxlY3Ryb2x5dGUgYXQgdGhlIGludGVyZmFjZS4gSGlnaCBsZXZlbHMgb2YgdHVyYmlkaXR5IGNhdXNlZCBieSBzdXNwZW5kZWQgc29saWRzIGhhdmUgYmVjb21lIGEgYmFycmllciB0byBkaXNzb2x1dGlvbiBvZiBOQnMgb3h5Z2VuIGludG8gdGhlIHdhdGVyIHNvbHV0aW9uLCBhbmQgdGh1cyBhZmZlY3RlZCB0aGUgdHJhbnNmZXIgcGVyZm9ybWFuY2UuIE9uIHRoZSBvdGhlciBoYW5kLCBieSBhcHBseWluZyBOQnMgZm9yIGp1c3QgYW4gaG91ciwgdXAgdG8gNy4yJSBkZWdyYWRhdGlvbiBvZiBnbHVjb3NlIGFzIHJlcHJlc2VudGF0aXZlIG9yZ2FuaWMgbWF0dGVyIHdhcyBhY2hpZXZlZC4gVGh1cywgTkJzIHRlY2hub2xvZ3kgd291bGQgbWFpbnRhaW4gYSBoaWdoIERPIGV4dGVudCBmb3IgYW4gZXh0ZW5kZWQgZHVyYXRpb24sIGFuZCB0aHVzIGNhbiBpbXByb3ZlIHdhdGVyIHF1YWxpdHkgcHJvdmlkZWQgdGhhdCB3YXRlciBjaGVtaXN0cnkgaXMgY2xvc2VseSBtb25pdG9yZWQgZHVyaW5nIGl0cyBhcHBsaWNhdGlvbi4gS2V5d29yZHM6IEFwcGxpY2F0aW9uIGNvbmRpdGlvbnMsIERpc3NvbHZlZCBveHlnZW4sIE5hbm9idWJibGVzLCBXYXRlciBxdWFsaXR5IiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjQ0OTEvZWVyLjIwMTYuMDI4IiwicmVzb3VyY2VVcmwiOiJodHRwOi8vZWVlci5vcmcvdXBsb2FkL2Vlci0yMS00LTQyNy5wZGYiLCJpc3N1ZWREYXRlIjoiMjAxNi0wOS0xMiIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkVudmlyb25tZW50YWwgRW5naW5lZXJpbmcgUmVzZWFyY2giLCJpc3N1ZSI6NCwidm9sdW1lIjoyMSwiaW1wYWN0RmFjdG9yIjoiNC4wNSJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjQyNywibGFzdFBhZ2UiOjQzNX19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJLb3JlYW4gU29jaWV0eSBvZiBFbnZpcm9ubWVudGFsIEVuZ2luZWVyaW5nIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjoyNCwiY2l0ZWRCeUNvdW50IjozM319LCJxdW90ZSI6IlRoaXMgdmVyaWZpZXMgdGhlIHBvc3NpYmlsaXR5IG9mIHRoZSBleGlzdGVuY2UgIG9mIE5Ccywgc2luY2UgYXQgYSBoaWdoZXIgRE8gY29uY2VudHJhdGlvbiwgdGhlIG94eWdlbiBkaWZmdXNpb24gIHdvdWxkIHRha2UgbW9yZSB0aW1lLiBUaGUgaGlnaCBnYXMgY29uY2VudHJhdGlvbiBpbiB0aGUgbGlxdWlkICBleHBsYWlucyB0aGUgZXh0ZW5zaW9uIG9mIE5CIHN0YWJpbGl0eS4gIFRoZSBtYWduaXR1ZGVzIG9mIHpldGEgcG90ZW50aWFsIG9mIG94eWdlbiBuYW5vLWJ1YmJsZWQgd2F0ZXIgIHdlcmUgcG9zc2libHkgaGlnaGVyIHRoYW4gdGhvc2Ugb2YgYWlyIG5hbm8tYnViYmxlZCB3YXRlci4gQXQgIGEgaGlnaCBtYWduaXR1ZGUgemV0YSBwb3RlbnRpYWwgdmFsdWUsIHRoZSBlbGVjdHJpY2FsbHkgY2hhcmdlZCAgcGFydGljbGVzIHRlbmQgdG8gcmVwZWwgZWFjaCBvdGhlciwgYXZvaWRpbmcgYWdncmVnYXRpb24uIEZvciBidWJibGUgIGRpc3BlcnNpb24sIHRoZSBoaWdoIHpldGEgcG90ZW50aWFsIGNvdWxkIGNyZWF0ZSByZXB1bHNpb24gZm9yY2VzICB0aGF0IHdvdWxkIGF2b2lkIHRoZSBjb2FsZXNjZW5jZSBvZiBidWJibGVzLCB0aHVzIGNvbnRyaWJ1dGluZyAgdG8gdGhlIHN0YWJpbGl6YXRpb24gb2YgdGhlIGJ1YmJsZXMuIFRoZXJlZm9yZSwgdGhlIG94eWdlbiBOQnMgIGNvdWxkIGJlIHN0YWJpbGl6ZWQgaW4gd2F0ZXIgYnkgdGhlIGVsZWN0cmljYWxseSBjaGFyZ2VkIHN1cmZhY2VzLCAgd2hpbGUgdGhlIG5hbm8tYnViYmxlZCBhaXIgd291bGQgYmUgYmVsb3cgdGhlIGxpbWl0IG9mIHRoZSBzdGFiaWxpemEtdGlvbiBieSByZXB1bHNpb24gZm9yY2VzLiBGdXJ0aGVybW9yZSwgdGhlIGJpZyBkaWZmZXJlbmNlIGJldHdlZW4gdGhlIHBlcmlvZHMgaW4gd2hpY2ggIGJ1YmJsZXMgY291bGQgYmUgb2JzZXJ2ZWQgYXMgc3RhYmxlIGluIHRoZSBveHlnZW4gbmFuby1idWJibGVkICB3YXRlciAoMTAtMTUgZCkgYW5kIGluIHRoYXQgb2YgYWlyIG5hbm8tYnViYmxlZCAoMy01IGQpIGlzIGxpa2VseSBGaWcuIDYuIEdyYXBocyBzaG93aW5nIHN0YWduYXRpb24gdGltZSBmb3IgZGlmZmVyZW50IGluaXRpYWwgZGlzc29sdmVkIG94eWdlbiAoRE8pLiByZWxhdGVkIHRvIHRoZSBsb3dlciBkaXNzb2x1dGlvbiBvZiB0aGUgYWlyIGluIHdhdGVyIGluIHJlbGF0aW9uIHRvIHRoZSBkaXNzb2x1dGlvbiBvZiBveHlnZW4uICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6NjYuMTc4ODAyNDkwMjM0MzgsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6NDAuNzE0ODg5NTI2MzY3MTksImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo2Ny41NzY3NzQ1OTcxNjc5NywibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42ODgyNzA1Njg4NDc2NTYsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo2OC45NjcyMzE3NTA0ODgyOCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NTcxODA3ODYxMzI4MSwiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjcwLjM1NzY4ODkwMzgwODYsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6MjUuMzQxMjU1MTg3OTg4MjgsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3MS43NTU2NTMzODEzNDc2NiwibGVmdCI6OS41NDk2NDE2MDkxOTE4OTUsIndpZHRoIjozOS4wMjQ3ODQwODgxMzQ3NjYsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3My4xNDYxMDI5MDUyNzM0NCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NTQwMjk4NDYxOTE0MDYsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3NC41MzY1NjAwNTg1OTM3NSwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NTM0NjUyNzA5OTYwOTQsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3NS45MzQ1MzIxNjU1MjczNCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NzMyMzY4NDY5MjM4MywiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjc3LjMyNDk4OTMxODg0NzY2LCJsZWZ0Ijo3Ljg2NTU0NTI3MjgyNzE0OCwid2lkdGgiOjQwLjY0OTM1MzAyNzM0Mzc1LCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6NzguNzE1NDQ2NDcyMTY3OTcsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6NDAuNjU0NjI0OTM4OTY0ODQ0LCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODAuMTEzNDEwOTQ5NzA3MDMsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6NDAuNjUxMDUwNTY3NjI2OTUsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo4MS41MDM4NjA0NzM2MzI4MSwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NTQzNDI2NTEzNjcxOSwiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjgyLjg5NDMxNzYyNjk1MzEyLCJsZWZ0Ijo3Ljg2NTU0NTI3MjgyNzE0OCwid2lkdGgiOjQwLjMyMTgyMzEyMDExNzE5LCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODQuMjkyMjg5NzMzODg2NzIsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6MTUuODcwMjk1NTI0NTk3MTY4LCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODUuNjgyNzQ2ODg3MjA3MDMsImxlZnQiOjkuNTQ5NjQxNjA5MTkxODk1LCJ3aWR0aCI6MzguOTg5NzA0MTMyMDgwMDgsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo4Ny4wNzMyMDQwNDA1MjczNCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC43MTg0OTQ0MTUyODMyLCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODguNDcxMTY4NTE4MDY2NCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42ODc0MTk4OTEzNTc0MiwiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjM0LjUxMzI3MTMzMTc4NzExLCJsZWZ0Ijo1MS45MzI3Nzc0MDQ3ODUxNTYsIndpZHRoIjo0MC42NDcwNTI3NjQ4OTI1OCwiaGVpZ2h0IjoyLjcxODY2NjU1MzQ5NzMxNDV9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjozOC43MjUxNTg2OTE0MDYyNSwibGVmdCI6NTEuOTMyNzc3NDA0Nzg1MTU2LCJ3aWR0aCI6NDAuNjQzMTM1MDcwODAwNzgsImhlaWdodCI6MS4yNTMyODg1MDc0NjE1NDc5fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6NDAuMTE1NjEyMDMwMDI5MywibGVmdCI6NTEuOTMyNzc3NDA0Nzg1MTU2LCJ3aWR0aCI6NDAuNjY0NTg1MTEzNTI1MzksImhlaWdodCI6MS4yNTMyODg1MDc0NjE1NDc5fX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNDMxIn19XQ%3D%3D. This not only supports aerobic respiration in fish and shrimp but also creates oxidative conditions conducive to pollutant degradation. Empirical evidence from shrimp ponds demonstrates that oxygen nanobubbles generated via systems like the aQua+75MO (168.9 nm average size, 1.04 × 10⁹ particles/mL at 1,000 L/h) can maintain stable DO levels while mitigating environmental stressors (Nghĩa et al., 2022)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5ndXnhu4VuIEjhu691IiwiZmFtaWx5IjoiTmdoxKlhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5ndXllbiBUaGkiLCJmYW1pbHkiOiJOZ3V5ZW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUGhhbiBUcuG7jW5nIiwiZmFtaWx5IjoiQsOsbmgifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTGUgVGhpIiwiZmFtaWx5IjoiTWF5In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRvbmcgVHJhbiIsImZhbWlseSI6Ikh1eSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQaOG6oW0gVGjDoWkiLCJmYW1pbHkiOiJHaWFuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTb3BoaWUiLCJmYW1pbHkiOiJTdOKAkEhpbGFpcmUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUGhhbiBUaGkiLCJmYW1pbHkiOiJWYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMjM0LTE3NDkiLCIyMjM0LTE3NTciXSwiaXNibiI6W10sImRvaSI6IjEwLjQ3ODUzL2Zhcy4yMDIyLmUzOSIsIm9wZW5hbGV4IjoiVzQyOTQ1OTc1NTQifSwidGl0bGUiOiJFZmZlY3Qgb2YgbmFub2J1YmJsZXMgKG94eWdlbiwgb3pvbmUpIG9uIHRoZSBQYWNpZmljIHdoaXRlIHNocmltcCAoUGVuYWV1cyB2YW5uYW1laSksIFZpYnJpbyBwYXJhaGFlbW9seXRpY3VzIGFuZCB3YXRlciBxdWFsaXR5IHVuZGVyIGxhYiBjb25kaXRpb25zIiwiYWJzdHJhY3QiOiJUaGlzIHN0dWR5IGFzc2Vzc2VkIHRoZSBlZmZlY3RzIG9mIG94eWdlbiBhbmQgb3pvbmUgbmFub2J1YmJsZXMgb24gZ2lsbCBtb3JwaG9sb2d5LCB3ZWlnaHQgZ2FpbiwgYW5kIG1vcnRhbGl0eSBvZiBQYWNpZmljIHdoaXRlIHNocmltcCAoUGVuYWV1cyB2YW5uYW1laSksIGFzIHdlbGwgYXMgdGhlIGxldmVsIG9mIFZpYnJpbyBwYXJhaGFlbW9seXRpY3VzIGFuZCB3YXRlciBxdWFsaXR5IG9mIHNocmltcCBjdWx0dXJlIHRhbmtzIHVuZGVyIGxhYiBjb25kaXRpb25zLiBUd28gZXhwZXJpbWVudHMgd2VyZSBjYXJyaWVkIG91dCB3aXRoIG94eWdlbiBtYWNyb2J1YmJsZSwgb3pvbmUgbWFjcm9idWJibGUsIG94eWdlbiBuYW5vYnViYmxlLCBvem9uZSBuYW5vYnViYmxlLCBhbmQgY29udHJvbCB0cmVhdG1lbnRzIChhaXItc3RvbmUgbWFjcm9idWJibGUpLiBFeHBlcmltZW50cyB3ZXJlIGRvbmUgaW4gdHJpcGxpY2F0ZSBpbiAxMDAgTCB0YW5rcyB3aXRoIDE14oCwIHNhbGluZSB3YXRlciwgYW5kIDIwIHNocmltcCBwZXIgdGFuay4gVGFua3MgaW4gRXhwZXJpbWVudCAxIHdlcmUgbm90IGlub2N1bGF0ZWQgd2l0aCBiYWN0ZXJpYTsgdGFua3MgaW4gRXhwZXJpbWVudCAyIHdlcmUgaW5vY3VsYXRlZCB3aXRoIFYuIHBhcmFoYWVtb2x5dGljdXMgYXQgYSBjb25jZW50cmF0aW9uIG9mIDEwNiBDRlUvbUwuIFRoZSByZXN1bHRzIHJldmVhbGVkIHRoYXQgc2hvcnQgdHJlYXRtZW50cyB3aXRoIG96b25lIG5hbm9idWJibGVzIGhhZCBtaW5pbWFsIGltcGFjdCBvbiBzaHJpbXAgZ2lsbHMsIG1vcnRhbGl0eSwgYW5kIGdyb3d0aCByYXRlcywgcmVkdWNlZCBWLiBwYXJhaGFlbW9seXRpY3VzIGNvbmNlbnRyYXRpb24gaW4gd2F0ZXIgY29tcGFyZWQgdG8gdGhlIG90aGVyIGdyb3VwcywgYW5kIGltcHJvdmUgd2F0ZXIgcXVhbGl0eS4gVGhlc2UgbGFib3JhdG9yeSByZXN1bHRzIGluZGljYXRlIHRoYXQgb3pvbmUgbmFub2J1YmJsZSB0cmVhdG1lbnQgbWF5IGJlIHVzZWZ1bCBmb3IgY29udHJvbGxpbmcgVi4gcGFyYWhhZW1vbHl0aWN1cy4gTW9yZSB3b3JrIGlzIG5lZWRlZCB0byBmaW5kIHRoZSBiZXN0IHByb3RvY29sIHRvIGFwcGx5IHRoZSB0ZWNobm9sb2d5IG9uIGEgY29tbWVyY2lhbCBzY2FsZS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuNDc4NTMvZmFzLjIwMjIuZTM5IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5lLWZhcy5vcmcvZG93bmxvYWQvZG93bmxvYWRfcGRmP3BpZD1mYXMtMjUtOC00MjkiLCJpc3N1ZWREYXRlIjoiMjAyMi0wOC0wMSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZpc2hlcmllcyBhbmQgYXF1YXRpYyBzY2llbmNlcyIsImlzc3VlIjo4LCJ2b2x1bWUiOjI1LCJpbXBhY3RGYWN0b3IiOiIxLjM0In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6NDI5LCJsYXN0UGFnZSI6NDQwfX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlNwcmluZ2VyIFNjaWVuY2UrQnVzaW5lc3MgTWVkaWEifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjMwLCJjaXRlZEJ5Q291bnQiOjEzfX0sInF1b3RlIjoiQSBuYW5vYnViYmxlIGdlbmVyYXRvciBtb2RlbCBhUXVhKzc1TU8gKEFxdWFQcm8gU29sdXRpb25zLCBTaW5nYXBvcmUsIFNpbmdhcG9yZSkgd2l0aCBhIDEgSFAgcG93ZXIgKDAuNzUga1cpIGh0dHBzOi8vZG9pLm9yZy8xMC40Nzg1My9GQVMuMjAyMi5lMzkgaHR0cHM6Ly93d3cuZS1mYXMub3JnIHwgNDMxIE5ndXllbiBIdXUgTmdoaWEsIGV0IGFsLiBGaXNoZXJpZXMgYW5kIEFxdWF0aWMgU2NpZW5jZXMgbW90b3IgYW5kIGEgd2F0ZXIgZmxvdyBvdXRwdXQgcmF0ZSBvZiAxLDAwMCBML2ggd2FzIHVzZWQgdG8gZ2VuZXJhdGUgbmFub2J1YmJsZXMuIFRoZSBtYWNoaW5lIHByb2R1Y2VkIG5hbm9idWJibGVzICB3aXRoIGFuIGF2ZXJhZ2Ugc2l6ZSBvZiAxNjguOSDCsSA3My44IG5tLCBhbmQgYSBjb25jZW50cmF0aW9uIG9mICAxLjA0IMOXIDEwIDkgwrEgMi42IMOXIDEwIDggcGFydGljbGVzL21MIChBcXVhUHJvIFNvbHV0aW9ucywgMjAxOSkuIFRvIGNyZWF0ZSBveHlnZW4gbmFub2J1YmJsZXMsIHdlIHVzZWQgYW4gb3h5Z2VuIGNvbmNlbnRyYXRvciAobW9kZWwgWXV3ZWxsIDdGLTEwLCBZdXdlbGwsIERhbnlhbmcsIENoaW5hOyAxMCBML21pbiBtYXggIGZsb3dyYXRlLCDiiaUgOTUuNSUgb3h5Z2VuIGNvbmNlbnRyYXRpb24sIGZsb3cgcmF0ZSBhdCA0IEwvbWluKSAgcHJvdmlkZWQgb3h5Z2VuIGZvciBuYW5vYnViYmxlIGdlbmVyYXRvci4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo4Ni4yMjE0MDUwMjkyOTY4OCwibGVmdCI6NTQuMDQ3NTUwMjAxNDE2MDE2LCJ3aWR0aCI6MzguMjAxMjQ4MTY4OTQ1MzEsImhlaWdodCI6MS42OTk2MzEzMzMzNTExMzUzfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6ODcuOTg1Mjk4MTU2NzM4MjgsImxlZnQiOjUxLjE5MDQwNjc5OTMxNjQwNiwid2lkdGgiOjQxLjA2MDEyMzQ0MzYwMzUxNiwiaGVpZ2h0IjoxLjY5OTYzMTMzMzM1MTEzNTN9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo5NC45ODc5Mzc5MjcyNDYxLCJsZWZ0Ijo4LjA5NTIzNTgyNDU4NDk2MSwid2lkdGgiOjg0LjI4NTcyMDgyNTE5NTMxLCJoZWlnaHQiOjEuNjYxNzUwNjc0MjQ3NzQxN319LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjMuNzI4NDYxMjY1NTYzOTY1LCJsZWZ0Ijo4LjEwNDc2MjA3NzMzMTU0Mywid2lkdGgiOjE2LjI1ODc2OTk4OTAxMzY3MiwiaGVpZ2h0IjoxLjI2NjgxMDUzNjM4NDU4MjV9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo0LjcxOTI3MjEzNjY4ODIzMiwibGVmdCI6NzAuNDM0NzIyOTAwMzkwNjIsIndpZHRoIjoxNC4wMzA2OTAxOTMxNzYyNywiaGVpZ2h0IjoxLjMwMDc2OTMyOTA3MTA0NX19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjEwLjM3NDIwNDYzNTYyMDExNywibGVmdCI6OC4wOTUyMzU4MjQ1ODQ5NjEsIndpZHRoIjo0MS4wNzg3NTgyMzk3NDYwOTQsImhlaWdodCI6Ni45OTEyOTg2NzU1MzcxMDl9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjoxNS41OTYyMjc2NDU4NzQwMjMsImxlZnQiOjE0LjE2NzcxNDExODk1NzUyLCJ3aWR0aCI6Ny42NDA0NTYxOTk2NDU5OTYsImhlaWdodCI6MS43NjkyNzUxODg0NDYwNDV9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjoxNS41OTYyMjc2NDU4NzQwMjMsImxlZnQiOjIxLjc3NzMwOTQxNzcyNDYxLCJ3aWR0aCI6MjcuMzc3NTMyOTU4OTg0Mzc1LCJoZWlnaHQiOjEuNzY5Mjc1MTg4NDQ2MDQ1fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MTcuNDI5NzU4MDcxODk5NDE0LCJsZWZ0Ijo4LjA5NTIzNTgyNDU4NDk2MSwid2lkdGgiOjQxLjA2MDMwNjU0OTA3MjI2NiwiaGVpZ2h0IjoxLjY5OTYzMzEyMTQ5MDQ3ODV9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjoxOS4xOTM2NDU0NzcyOTQ5MjIsImxlZnQiOjguMDk1MjM1ODI0NTg0OTYxLCJ3aWR0aCI6NDEuMDY5ODY5OTk1MTE3MTksImhlaWdodCI6MS42OTk2MzMxMjE0OTA0Nzg1fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MjAuOTU3NTM0NzkwMDM5MDYyLCJsZWZ0Ijo4LjA5NTIzNTgyNDU4NDk2MSwid2lkdGgiOjQxLjA1ODc3Njg1NTQ2ODc1LCJoZWlnaHQiOjEuNjk5NjMzMTIxNDkwNDc4NX19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjIyLjcyMTQyNDEwMjc4MzIwMywibGVmdCI6OC4wOTUyMzU4MjQ1ODQ5NjEsIndpZHRoIjo0MC43MTQyOTA2MTg4OTY0ODQsImhlaWdodCI6MS42OTk2MzMxMjE0OTA0Nzg1fX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNDMwIn19XQ%3D%3D.

The primary mechanism for water quality improvement lies in nanobubbles' exceptional gas dissolution kinetics, governed by enhanced mass transfer coefficients that surpass coarse bubble diffusers by factors of 2-5 (Svetovoy, 2021)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlYuIEIuIiwiZmFtaWx5IjoiU3ZldG92b3kifX1dLCJpZHMiOnsiaXNzbiI6WyIxMzU5LTAyOTQiLCIxODc5LTAzOTkiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMTYvai5jb2Npcy4yMDIxLjEwMTQyMyIsIm1hZyI6IjMxMjY0ODQ4NDgiLCJvcGVuYWxleCI6IlczMTI2NDg0ODQ4In0sInRpdGxlIjoiU3BvbnRhbmVvdXMgY2hlbWljYWwgcmVhY3Rpb25zIGJldHdlZW4gaHlkcm9nZW4gYW5kIG94eWdlbiBpbiBuYW5vYnViYmxlcyIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2ouY29jaXMuMjAyMS4xMDE0MjMiLCJyZXNvdXJjZVVybCI6Imh0dHA6Ly9hcnhpdi5vcmcvcGRmLzIxMDIuMDMxMjYiLCJpc3N1ZWREYXRlIjoiMjAyMS0wMi0wNSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkN1cnJlbnQgT3BpbmlvbiBpbiBDb2xsb2lkICYgSW50ZXJmYWNlIFNjaWVuY2UiLCJ2b2x1bWUiOjUyfSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjoxMDE0MjMsImxhc3RQYWdlIjoxMDE0MjN9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRWxzZXZpZXIgQlYifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjYxLCJjaXRlZEJ5Q291bnQiOjJ9fSwicXVvdGUiOiJUaGlzIGF0dGVudGlvbiBpcyBkcml2ZW4gYnkgYSBmdW5kYS1tZW50YWwgcHJvYmxlbTogTkJzIGxpdmUgbXVjaCB0b28gbG9uZyBpbiBjb21wYXJpLXNvbiB3aXRoIHRoZSBwcmVkaWN0aW9uIGdpdmVuIGJ5IHRoZSBkaWZmdXNpb24gZGlzc29sdS10aW9uIHRoZW9yeSBbNCwgNV0uIE9uIHRoZSBvdGhlciBoYW5kLCBleGNpdGluZyBhcHBsaWNhLXRpb25zIG9mIE5CcyBpbiBlbnZpcm9ubWVudCBbNiwgN10sIGFncmljdWx0dXJlIFs4XSwgZmxvdGEtdGlvbiBbOSwgMTBdLCBoZWFsdGhjYXJlIFsxMSwgMTJdLCBhbmQgY2hlbWlzdHJ5IFsxM10gaGF2ZSBlbWVyZ2VkIHRvIG5hbWUgb25seSBhIGZldy4gSW4gY29udHJhc3Qgd2l0aCBzdXJmYWNlIE5Ccywgd2hpY2ggZXhpc3Qgb24gdGhlIHNvbGlkLWxpcXVpZCBpbnRlcmZhY2UsIGJ1bGsgTkJzIGZpbGwgdGhlIHZvbHVtZSBvZiBsaXF1aWQuIFRoZXNlIE5CcyBhcmUgZ2VuZXJhdGVkIGJ5IG1vc3RseSBtZWNoYW5pY2FsIGluamVjdGlvbiBvZiBkaWYtZmVyZW50IGdhc2VzIFs5XSBvciBieSB1bHRyYXNvbmljIGNhdml0YXRpb24gWzEwXSwgYnV0IGh5LWRyb2dlbiBhbmQgb3h5Z2VuIE5CcyB3aGVyZSBhbHNvIGdlbmVyYXRlZCBieSBEQyBlbGVjdHJvbHlzaXMgWzE0LCAxNV0uIExhdGVyIHRoZXNlIG1ldGhvZHMgd2lsbCBiZSBjYWxsZWQgdGhlIHJlZ3VsYXIgbWV0aG9kcy4gVGhlIHNpemUgZGlzdHJpYnV0aW9uIG9mIHRoZSBOQnMgd2FzIGNoYXJhY3Rlcml6ZWQgYnkgdGhlIGR5bmFtaWMgbGlnaHQgc2NhdHRlcmluZyBhbmQgaW5kaXZpZHVhbCBidWJibGVzIHdlcmUgb2JzZXJ2ZWQgYnkgZWxlY3Ryb24gbWljcm9zY29weSBmcm9tIGZyZWV6ZSBmcmFjdHVyZSByZXBsaWNhcyBbMTZdLCBieSBwaGFzZSBtaWNyb3Njb3B5IGFuZCBwb2xhcmltZXRyaWMgc2NhdHRlcm9tZXRyeSBbMTddLiBVbmV4cGVjdGVkbHksIE5CcyBjb250YWluaW5nIG94eWdlbiBhcmUgYWJsZSB0byBwcm8tZHVjZSByZWFjdGl2ZSBveHlnZW4gc3BlY2llcyB3aGljaCBjYW4gb3hpZGl6ZSBwb2xsdS10YW50cyBhbmQgcGF0aG9nZW5zIGluIHdhdGVyIFsxOOKAkzIwXSwgd2hpbGUgc2VwYXJhdGUgb3h5LWdlbiBtb2xlY3VsZXMgaGF2ZSBubyBlZmZlY3Qgb24gb3JnYW5pY3MuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6MzYuOTUwNjQ1NDQ2Nzc3MzQ0LCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA1MjI2MTM1MjUzOTA2LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjM4LjM5NzIzNTg3MDM2MTMzLCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA0ODk5OTc4NjM3Njk1LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjM5Ljg0MzgyNjI5Mzk0NTMxLCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA1MjI2MTM1MjUzOTA2LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjQxLjI5MDQxNjcxNzUyOTMsImxlZnQiOjguODIzNTI1NDI4NzcxOTczLCJ3aWR0aCI6NDAuMDUzODgyNTk4ODc2OTUsImhlaWdodCI6MS4yNTc5MDU5NjAwODMwMDc4fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NDIuNzM3MDEwOTU1ODEwNTUsImxlZnQiOjguODIzNTI1NDI4NzcxOTczLCJ3aWR0aCI6NDAuMDUyMjQ2MDkzNzUsImhlaWdodCI6MS4yNTc5MDU5NjAwODMwMDc4fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NDQuMTgzNjAxMzc5Mzk0NTMsImxlZnQiOjguODIzNTI1NDI4NzcxOTczLCJ3aWR0aCI6NDAuMDUyMjQ2MDkzNzUsImhlaWdodCI6MS4yNTc5MDU5NjAwODMwMDc4fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NDUuNjMwMTg3OTg4MjgxMjUsImxlZnQiOjguODIzNTI1NDI4NzcxOTczLCJ3aWR0aCI6MjAuNTMyMzk4MjIzODc2OTUzLCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjQ3LjA3Njc4MjIyNjU2MjUsImxlZnQiOjEwLjQ1MTQ2NjU2MDM2Mzc3LCJ3aWR0aCI6MzguNDE5NDk4NDQzNjAzNTE2LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjQ4LjUyMzM3MjY1MDE0NjQ4NCwibGVmdCI6OC44MjM1MjU0Mjg3NzE5NzMsIndpZHRoIjo0MC4wNTM4ODY0MTM1NzQyMiwiaGVpZ2h0IjoxLjI1NzkwNTk2MDA4MzAwNzh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo0OS45Njk5NjY4ODg0Mjc3MzQsImxlZnQiOjguODIzNTI1NDI4NzcxOTczLCJ3aWR0aCI6NDAuMDQ3Mzc0NzI1MzQxOCwiaGVpZ2h0IjoxLjI1NzkwNTk2MDA4MzAwNzh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo1MS40MTY1NTczMTIwMTE3MiwibGVmdCI6OC44MjM1MjU0Mjg3NzE5NzMsIndpZHRoIjo0MC4wNTIyNDYwOTM3NSwiaGVpZ2h0IjoxLjI1NzkwNTk2MDA4MzAwNzh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo1Mi44NjMxNDM5MjA4OTg0NCwibGVmdCI6OC44MjM1MjU0Mjg3NzE5NzMsIndpZHRoIjo0MC4wNDczNzQ3MjUzNDE4LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjU0LjMwOTczODE1OTE3OTY5LCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA0NDExMzE1OTE3OTY5LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjU1Ljc1NjMzMjM5NzQ2MDk0LCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA0OTAxMTIzMDQ2ODc1LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjU3LjIwMjkxOTAwNjM0NzY1NiwibGVmdCI6OC44MjM1MjU0Mjg3NzE5NzMsIndpZHRoIjo0MC4wNDczNzQ3MjUzNDE4LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjU4LjY0OTUwOTQyOTkzMTY0LCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA0OTAwMzYwMTA3NDIyLCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjYwLjA5NjEwMzY2ODIxMjg5LCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA0NTc0NTg0OTYwOTM3NSwiaGVpZ2h0IjoxLjI1NzkwNTk2MDA4MzAwNzh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2MS41NDI2OTAyNzcwOTk2MSwibGVmdCI6OC44MjM1MjU0Mjg3NzE5NzMsIndpZHRoIjoyNS43MTIzMDMxNjE2MjEwOTQsImhlaWdodCI6MS4yNTc5MDU5NjAwODMwMDc4fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NjIuOTg5NDA2NTg1NjkzMzYsImxlZnQiOjEwLjQ1MTQ2NjU2MDM2Mzc3LCJ3aWR0aCI6MzguNDIxMTEyMDYwNTQ2ODc1LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjY0LjQzNTk5NzAwOTI3NzM0LCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA0NTc0NTg0OTYwOTM3NSwiaGVpZ2h0IjoxLjI1NzkwNTk2MDA4MzAwNzh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2NS44ODI1OTEyNDc1NTg2LCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA0ODk5NTk3MTY3OTY5LCJoZWlnaHQiOjEuMjU3OTAyMTQ1Mzg1NzQyMn19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjY3LjMyOTE5MzExNTIzNDM4LCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjMwLjY2OTE3OTkxNjM4MTgzNiwiaGVpZ2h0IjoxLjI1NzkwMjE0NTM4NTc0MjJ9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIxMDE0MjMifX1d. Unlike microbubbles, which rise rapidly and burst at the surface, nanobubbles exhibit a reduced terminal velocity according to Stokes' law (v ∝ r²), ensuring uniform oxygen distribution throughout the water column. Studies in Penaeus vannamei culture have shown nanobubble aeration elevating DO from baseline levels, correlating with reduced total ammonia nitrogen by approximately 9% (from 2.58 mg/L in controls to 2.35 mg/L in treatments) over rearing cycles (Susanti et al., 2021)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkxpbHkiLCJmYW1pbHkiOiJTdXNhbnRpIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlN1eXVkIFdhcm5vIiwiZmFtaWx5IjoiVXRvbW8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTm92ZXJpdGEgRGlhbiIsImZhbWlseSI6IlRha2FyaW5hIn19XSwiaWRzIjp7Imlzc24iOlsiMjExNy00NDU4IiwiMjI3My0xNzA5Il0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDUxL2Jpb2NvbmYvMjAyMTMzMDUwMDUiLCJtYWciOiIzMTkzNTYzNzAzIiwib3BlbmFsZXgiOiJXMzE5MzU2MzcwMyJ9LCJ0aXRsZSI6IlN1c3RhaW5hYmlsaXR5IGFuZCBmZWFzaWJpbGl0eSBhc3Nlc3NtZW50cyBvZiBuYW5vYnViYmxlIGFlcmF0aW9uIHRlY2hub2xvZ3kgaW4gZWNvbm9taWMtc29jaW8gZW52aXJvbm1lbnQgb2YgUGVuYWV1cyB2YW5uYW1laSBzaHJpbXAgZmFybWluZyIsImFic3RyYWN0IjoiUGVuYWV1cyB2YW5uYW1laSBzaHJpbXAgZmFybWluZyBpbiBJbmRvbmVzaWEgaXMgZmFjZWQgd2l0aCBzZXZlcmFsIGNoYWxsZW5nZXMgaW4gdGhlIGVudmlyb25tZW50YWwsIGVjb25vbWljLCBhbmQgc29jaWFsIGFzcGVjdHMuIFRoZXJlZm9yZSwgdGhpcyBzdHVkeSBhaW1zIHRvIGFzc2VzcyB0aGUgYmVuZWZpdHMgb2Ygbm92ZWwgbmFub2J1YmJsZSBhZXJhdGlvbiBzeXN0ZW1zIGZvciB0aGUgZWNvc3lzdGVtLCBidXNpbmVzc2VzLCBhbmQgY29tbXVuaXRpZXMuIFRoaXMgaXMgYW4gZXhwZXJpbWVudGFsIHN0dWR5IGNvbmR1Y3RlZCBieSBjb21wYXJpbmcgUC4gdmFubmFtZWkgcG9zdCBsYXJ2YSAxMCByZWFyZWQgYXQgYSBkZW5zaXR5IG9mIDIwMDAgaW5kcy4vTCBpbiBhIHBvbmQgdHJlYXRlZCB3aXRoIG5hbm9idWJibGUgYW5kIHdpdGhvdXQgdHJlYXRtZW50cyAoY29udHJvbCkuIEZ1cnRoZXJtb3JlLCB0aGUgTmV0IFByZXNlbnQgVmFsdWUgKE5QVikgYW5kIHBheWJhY2sgcGVyaW9kIHdlcmUgY2FsY3VsYXRlZCB0byBhc3Nlc3MgdGhlIGVjb25vbWljIGZlYXNpYmlsaXR5IG9mIG5hbm9idWJibGUsIHdoaWxlIGNvbW11bml0eSBpbnRlcnZpZXdzIHdlcmUgdXNlZCBmb3IgdGhlIHNvY2lhbCBzZWN0b3JzLiBCYXNlZCBvbiB0aGUgcmVzdWx0cywgZW52aXJvbm1lbnRhbCB3YXN0ZXMgb2Ygc2hyaW1wIGZhcm1pbmcgaW4gZm9ybSBvZiB0b3RhbCBhbW1vbmlhIG5pdHJvZ2VuIChUQU4pIHdlcmUgcmVkdWNlZCBieSA5JSBmcm9tIDIuNTggbWcvbCAoOTUlQ0k6IDAsOTEsIDQsMjUpIGluIGNvbnRyb2wgY29tcGFyZWQgdG8gMi4zNSBtZy9sICg5NSVDSTogMCw4NiwgMyw4NCkgaW4gdHJlYXRtZW50LiBGdXJ0aGVybW9yZSwgdGhlIHJldmVudWVzIGZyb20gcG9zdCBsYXJ2YSBzYWxlcyBmb3IgZml2ZSB5ZWFycyBhbmQgbmFub2J1YmJsZSBpbnZlc3RtZW50IGNvc3RzIHNob3dlZCB0aGF0IHRoZSBlc3RpbWF0ZWQgTlBWIHdhcyBJRFIgNjQsODI0LDM3NCB3aXRoIGEgcGF5YmFjayBwZXJpb2Qgb2YgMS43IHllYXJzLiBUaGUgaW50ZXJ2aWV3cyBvbiB0cmFkaXRpb25hbCBzaHJpbXAgZmFybWluZyBzaG93ZWQgdGhhdCA2MS4xJSB0byA3Mi4yJSBvZiBjb21tdW5pdHkgbWVtYmVycyBhZ3JlZWQgb24gdGhlIHVzZSBvZiBuYW5vYnViYmxlIGFlcmF0aW9uIHRvIHN1cHBvcnQgbGl2ZWxpaG9vZHMuIFRoZXJlZm9yZSwgdGhlIHVzZSBvZiBuYW5vYnViYmxlcyBpcyBmZWFzaWJsZSB0byBzdXBwb3J0IHN1c3RhaW5hYmxlIFAuIHZhbm5hbWVpIGZhcm1pbmcuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjEwNTEvYmlvY29uZi8yMDIxMzMwNTAwNSIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cuYmlvLWNvbmZlcmVuY2VzLm9yZy9hcnRpY2xlcy9iaW9jb25mL3BkZi8yMDIxLzA1L2Jpb2NvbmZfaWNhdmVzczIwMjFfMDUwMDUucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjEtMDEtMDEiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA5IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJCSU8gV2ViIG9mIENvbmZlcmVuY2VzIiwidm9sdW1lIjozMywiaW1wYWN0RmFjdG9yIjoiMC42MiJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjUwMDUsImxhc3RQYWdlIjo1MDA1fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkVEUCBTY2llbmNlcyJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MTEsImNpdGVkQnlDb3VudCI6NH19LCJxdW90ZSI6IkFic3RyYWN0LiBQZW5hZXVzIHZhbm5hbWVpIHNocmltcCBmYXJtaW5nIGluIEluZG9uZXNpYSBpcyBmYWNlZCB3aXRoIHNldmVyYWwgY2hhbGxlbmdlcyBpbiB0aGUgZW52aXJvbm1lbnRhbCwgZWNvbm9taWMsIGFuZCBzb2NpYWwgYXNwZWN0cy4gIFRoZXJlZm9yZSwgdGhpcyBzdHVkeSBhaW1zIHRvIGFzc2VzcyB0aGUgYmVuZWZpdHMgb2Ygbm92ZWwgbmFub2J1YmJsZSAgYWVyYXRpb24gc3lzdGVtcyBmb3IgdGhlIGVjb3N5c3RlbSwgYnVzaW5lc3NlcywgYW5kIGNvbW11bml0aWVzLiBUaGlzIGlzIGFuICBleHBlcmltZW50YWwgc3R1ZHkgY29uZHVjdGVkIGJ5IGNvbXBhcmluZyBQLiB2YW5uYW1laSBwb3N0IGxhcnZhIDEwICByZWFyZWQgYXQgYSBkZW5zaXR5IG9mIDIwMDAgaW5kcy4vTCBpbiBhIHBvbmQgdHJlYXRlZCB3aXRoIG5hbm9idWJibGUgYW5kICB3aXRob3V0IHRyZWF0bWVudHMgKGNvbnRyb2wpLiBGdXJ0aGVybW9yZSwgdGhlIE5ldCBQcmVzZW50IFZhbHVlIChOUFYpIGFuZCAgcGF5YmFjayBwZXJpb2Qgd2VyZSBjYWxjdWxhdGVkIHRvIGFzc2VzcyB0aGUgZWNvbm9taWMgZmVhc2liaWxpdHkgb2YgIG5hbm9idWJibGUsIHdoaWxlIGNvbW11bml0eSBpbnRlcnZpZXdzIHdlcmUgdXNlZCBmb3IgdGhlIHNvY2lhbCBzZWN0b3JzLiAgQmFzZWQgb24gdGhlIHJlc3VsdHMsIGVudmlyb25tZW50YWwgd2FzdGVzIG9mIHNocmltcCBmYXJtaW5nIGluIGZvcm0gb2YgIHRvdGFsIGFtbW9uaWEgbml0cm9nZW4gKFRBTikgd2VyZSByZWR1Y2VkIGJ5IDklIGZyb20gMi41OCBtZy9sICg5NSVDSTogIDAsOTEsIDQsMjUpIGluIGNvbnRyb2wgY29tcGFyZWQgdG8gMi4zNSBtZy9sICg5NSVDSTogMCw4NiwgMyw4NCkgaW4gIHRyZWF0bWVudC4gRnVydGhlcm1vcmUsIHRoZSByZXZlbnVlcyBmcm9tIHBvc3QgbGFydmEgc2FsZXMgZm9yIGZpdmUgeWVhcnMgYW5kICBuYW5vYnViYmxlIGludmVzdG1lbnQgY29zdHMgc2hvd2VkIHRoYXQgdGhlIGVzdGltYXRlZCBOUFYgd2FzIElEUiAgNjQsODI0LDM3NCB3aXRoIGEgcGF5YmFjayBwZXJpb2Qgb2YgMS43IHllYXJzLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjQ1LjQ4MzI4NzgxMTI3OTMsImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2LjgwMDU2NzYyNjk1MzEyNSwiaGVpZ2h0IjoxLjc0OTIzODYxMDI2NzYzOTJ9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo0Ny4xNjcxNzE0NzgyNzE0ODQsImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2Ljc3NTAzOTY3Mjg1MTU2LCJoZWlnaHQiOjEuNDA2Nzg1OTY0OTY1ODIwM319LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjQ4LjYyMzgwOTgxNDQ1MzEyNSwibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuNzcxMzAxMjY5NTMxMjUsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NTAuMDgwNDQ4MTUwNjM0NzY2LCJsZWZ0IjoyMS44Mjc3NzAyMzMxNTQyOTcsIndpZHRoIjo1Ni44MjAyOTcyNDEyMTA5NCwiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo1MS41MzcwOTAzMDE1MTM2NywibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuODI2NTMwNDU2NTQyOTcsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NTMuMDEwNjY1ODkzNTU0NjksImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2Ljc2NTcyNDE4MjEyODkwNiwiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo1NC40NjczMDQyMjk3MzYzMywibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuNzkzNzEyNjE1OTY2OCwiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo1NS45MjM5NDYzODA2MTUyMzQsImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2Ljc5NTYxNjE0OTkwMjM0NCwiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo1Ny4zODA1ODg1MzE0OTQxNCwibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuNzc2OTI3OTQ3OTk4MDUsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NTguODU0MTY0MTIzNTM1MTU2LCJsZWZ0IjoyMS44Mjc3NzAyMzMxNTQyOTcsIndpZHRoIjo1Ni43ODA2NjYzNTEzMTgzNiwiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2MC4zMTA3OTg2NDUwMTk1MywibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuODMzMzc0MDIzNDM3NSwiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2MS43Njc0NDA3OTU4OTg0NCwibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuNzQ3MDA5Mjc3MzQzNzUsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NjMuMjI4MzEzNDQ2MDQ0OTIsImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2Ljc4ODEyNzg5OTE2OTkyLCJoZWlnaHQiOjEuNDA2Nzg1OTY0OTY1ODIwM319LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjY0LjcwMTg4OTAzODA4NTk0LCJsZWZ0IjoyMS44Mjc3NzAyMzMxNTQyOTcsIndpZHRoIjo1Ni43ODYyNjI1MTIyMDcwMywiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2Ni4xNTg1MzExODg5NjQ4NCwibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuODE1MzE5MDYxMjc5MywiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI1MDA1In19XQ%3D%3D. This DO enrichment also suppresses anaerobic zones, minimizing nitrite and nitrate buildup via optimized nitrification, thus stabilizing pH and alkalinity critical for biofloc systems.

Reactive oxygen species generated at the nanobubble interface further amplify remediation efficacy through advanced oxidation processes. Under elevated Laplace pressure (ΔP = 2σ/r), where internal pressures exceed several atmospheres for 100 nm bubbles, interfacial reactions produce hydroxyl radicals (•OH), superoxide (O₂⁻•), and hydrogen peroxide (H₂O₂) (Hutagalung et al., 2023; Svetovoy, 2021)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlYuIEIuIiwiZmFtaWx5IjoiU3ZldG92b3kifX1dLCJpZHMiOnsiaXNzbiI6WyIxMzU5LTAyOTQiLCIxODc5LTAzOTkiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMTYvai5jb2Npcy4yMDIxLjEwMTQyMyIsIm1hZyI6IjMxMjY0ODQ4NDgiLCJvcGVuYWxleCI6IlczMTI2NDg0ODQ4In0sInRpdGxlIjoiU3BvbnRhbmVvdXMgY2hlbWljYWwgcmVhY3Rpb25zIGJldHdlZW4gaHlkcm9nZW4gYW5kIG94eWdlbiBpbiBuYW5vYnViYmxlcyIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2ouY29jaXMuMjAyMS4xMDE0MjMiLCJyZXNvdXJjZVVybCI6Imh0dHA6Ly9hcnhpdi5vcmcvcGRmLzIxMDIuMDMxMjYiLCJpc3N1ZWREYXRlIjoiMjAyMS0wMi0wNSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkN1cnJlbnQgT3BpbmlvbiBpbiBDb2xsb2lkICYgSW50ZXJmYWNlIFNjaWVuY2UiLCJ2b2x1bWUiOjUyfSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjoxMDE0MjMsImxhc3RQYWdlIjoxMDE0MjN9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRWxzZXZpZXIgQlYifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjYxLCJjaXRlZEJ5Q291bnQiOjJ9fSwicXVvdGUiOiJUaGlzIGF0dGVudGlvbiBpcyBkcml2ZW4gYnkgYSBmdW5kYS1tZW50YWwgcHJvYmxlbTogTkJzIGxpdmUgbXVjaCB0b28gbG9uZyBpbiBjb21wYXJpLXNvbiB3aXRoIHRoZSBwcmVkaWN0aW9uIGdpdmVuIGJ5IHRoZSBkaWZmdXNpb24gZGlzc29sdS10aW9uIHRoZW9yeSBbNCwgNV0uIE9uIHRoZSBvdGhlciBoYW5kLCBleGNpdGluZyBhcHBsaWNhLXRpb25zIG9mIE5CcyBpbiBlbnZpcm9ubWVudCBbNiwgN10sIGFncmljdWx0dXJlIFs4XSwgZmxvdGEtdGlvbiBbOSwgMTBdLCBoZWFsdGhjYXJlIFsxMSwgMTJdLCBhbmQgY2hlbWlzdHJ5IFsxM10gaGF2ZSBlbWVyZ2VkIHRvIG5hbWUgb25seSBhIGZldy4gSW4gY29udHJhc3Qgd2l0aCBzdXJmYWNlIE5Ccywgd2hpY2ggZXhpc3Qgb24gdGhlIHNvbGlkLWxpcXVpZCBpbnRlcmZhY2UsIGJ1bGsgTkJzIGZpbGwgdGhlIHZvbHVtZSBvZiBsaXF1aWQuIFRoZXNlIE5CcyBhcmUgZ2VuZXJhdGVkIGJ5IG1vc3RseSBtZWNoYW5pY2FsIGluamVjdGlvbiBvZiBkaWYtZmVyZW50IGdhc2VzIFs5XSBvciBieSB1bHRyYXNvbmljIGNhdml0YXRpb24gWzEwXSwgYnV0IGh5LWRyb2dlbiBhbmQgb3h5Z2VuIE5CcyB3aGVyZSBhbHNvIGdlbmVyYXRlZCBieSBEQyBlbGVjdHJvbHlzaXMgWzE0LCAxNV0uIExhdGVyIHRoZXNlIG1ldGhvZHMgd2lsbCBiZSBjYWxsZWQgdGhlIHJlZ3VsYXIgbWV0aG9kcy4gVGhlIHNpemUgZGlzdHJpYnV0aW9uIG9mIHRoZSBOQnMgd2FzIGNoYXJhY3Rlcml6ZWQgYnkgdGhlIGR5bmFtaWMgbGlnaHQgc2NhdHRlcmluZyBhbmQgaW5kaXZpZHVhbCBidWJibGVzIHdlcmUgb2JzZXJ2ZWQgYnkgZWxlY3Ryb24gbWljcm9zY29weSBmcm9tIGZyZWV6ZSBmcmFjdHVyZSByZXBsaWNhcyBbMTZdLCBieSBwaGFzZSBtaWNyb3Njb3B5IGFuZCBwb2xhcmltZXRyaWMgc2NhdHRlcm9tZXRyeSBbMTddLiBVbmV4cGVjdGVkbHksIE5CcyBjb250YWluaW5nIG94eWdlbiBhcmUgYWJsZSB0byBwcm8tZHVjZSByZWFjdGl2ZSBveHlnZW4gc3BlY2llcyB3aGljaCBjYW4gb3hpZGl6ZSBwb2xsdS10YW50cyBhbmQgcGF0aG9nZW5zIGluIHdhdGVyIFsxOOKAkzIwXSwgd2hpbGUgc2VwYXJhdGUgb3h5LWdlbiBtb2xlY3VsZXMgaGF2ZSBubyBlZmZlY3Qgb24gb3JnYW5pY3MuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6MzYuOTUwNjQ1NDQ2Nzc3MzQ0LCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA1MjI2MTM1MjUzOTA2LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjM4LjM5NzIzNTg3MDM2MTMzLCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA0ODk5OTc4NjM3Njk1LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjM5Ljg0MzgyNjI5Mzk0NTMxLCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA1MjI2MTM1MjUzOTA2LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjQxLjI5MDQxNjcxNzUyOTMsImxlZnQiOjguODIzNTI1NDI4NzcxOTczLCJ3aWR0aCI6NDAuMDUzODgyNTk4ODc2OTUsImhlaWdodCI6MS4yNTc5MDU5NjAwODMwMDc4fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NDIuNzM3MDEwOTU1ODEwNTUsImxlZnQiOjguODIzNTI1NDI4NzcxOTczLCJ3aWR0aCI6NDAuMDUyMjQ2MDkzNzUsImhlaWdodCI6MS4yNTc5MDU5NjAwODMwMDc4fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NDQuMTgzNjAxMzc5Mzk0NTMsImxlZnQiOjguODIzNTI1NDI4NzcxOTczLCJ3aWR0aCI6NDAuMDUyMjQ2MDkzNzUsImhlaWdodCI6MS4yNTc5MDU5NjAwODMwMDc4fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NDUuNjMwMTg3OTg4MjgxMjUsImxlZnQiOjguODIzNTI1NDI4NzcxOTczLCJ3aWR0aCI6MjAuNTMyMzk4MjIzODc2OTUzLCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjQ3LjA3Njc4MjIyNjU2MjUsImxlZnQiOjEwLjQ1MTQ2NjU2MDM2Mzc3LCJ3aWR0aCI6MzguNDE5NDk4NDQzNjAzNTE2LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjQ4LjUyMzM3MjY1MDE0NjQ4NCwibGVmdCI6OC44MjM1MjU0Mjg3NzE5NzMsIndpZHRoIjo0MC4wNTM4ODY0MTM1NzQyMiwiaGVpZ2h0IjoxLjI1NzkwNTk2MDA4MzAwNzh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo0OS45Njk5NjY4ODg0Mjc3MzQsImxlZnQiOjguODIzNTI1NDI4NzcxOTczLCJ3aWR0aCI6NDAuMDQ3Mzc0NzI1MzQxOCwiaGVpZ2h0IjoxLjI1NzkwNTk2MDA4MzAwNzh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo1MS40MTY1NTczMTIwMTE3MiwibGVmdCI6OC44MjM1MjU0Mjg3NzE5NzMsIndpZHRoIjo0MC4wNTIyNDYwOTM3NSwiaGVpZ2h0IjoxLjI1NzkwNTk2MDA4MzAwNzh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo1Mi44NjMxNDM5MjA4OTg0NCwibGVmdCI6OC44MjM1MjU0Mjg3NzE5NzMsIndpZHRoIjo0MC4wNDczNzQ3MjUzNDE4LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjU0LjMwOTczODE1OTE3OTY5LCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA0NDExMzE1OTE3OTY5LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjU1Ljc1NjMzMjM5NzQ2MDk0LCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA0OTAxMTIzMDQ2ODc1LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjU3LjIwMjkxOTAwNjM0NzY1NiwibGVmdCI6OC44MjM1MjU0Mjg3NzE5NzMsIndpZHRoIjo0MC4wNDczNzQ3MjUzNDE4LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjU4LjY0OTUwOTQyOTkzMTY0LCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA0OTAwMzYwMTA3NDIyLCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjYwLjA5NjEwMzY2ODIxMjg5LCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA0NTc0NTg0OTYwOTM3NSwiaGVpZ2h0IjoxLjI1NzkwNTk2MDA4MzAwNzh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2MS41NDI2OTAyNzcwOTk2MSwibGVmdCI6OC44MjM1MjU0Mjg3NzE5NzMsIndpZHRoIjoyNS43MTIzMDMxNjE2MjEwOTQsImhlaWdodCI6MS4yNTc5MDU5NjAwODMwMDc4fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NjIuOTg5NDA2NTg1NjkzMzYsImxlZnQiOjEwLjQ1MTQ2NjU2MDM2Mzc3LCJ3aWR0aCI6MzguNDIxMTEyMDYwNTQ2ODc1LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjY0LjQzNTk5NzAwOTI3NzM0LCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA0NTc0NTg0OTYwOTM3NSwiaGVpZ2h0IjoxLjI1NzkwNTk2MDA4MzAwNzh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2NS44ODI1OTEyNDc1NTg2LCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA0ODk5NTk3MTY3OTY5LCJoZWlnaHQiOjEuMjU3OTAyMTQ1Mzg1NzQyMn19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjY3LjMyOTE5MzExNTIzNDM4LCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjMwLjY2OTE3OTkxNjM4MTgzNiwiaGVpZ2h0IjoxLjI1NzkwMjE0NTM4NTc0MjJ9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIxMDE0MjMifX0seyJvcmlnaW4iOjQsIndvcmsiOnsidHlwZSI6OCwic3VidHlwZSI6eyJvbmVvZktpbmQiOiJhcnRpY2xlVHlwZSIsImFydGljbGVUeXBlIjoxfSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU3V0cmlzbm8gU2Fsb21vIiwiZmFtaWx5IjoiSHV0YWdhbHVuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBbmRlIEZ1ZGphIiwiZmFtaWx5IjoiUmFmcnlhbnRvIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IldlaSIsImZhbWlseSI6IlN1biJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJOdXJvY2htYSIsImZhbWlseSI6Ikp1bGlhc2loIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNyaSIsImZhbWlseSI6IkFkaXRpYSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJKaXpob3UiLCJmYW1pbHkiOiJKaWFuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBcnJhbWVsIiwiZmFtaWx5IjoiQXJyYW1lbCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJIZXJtYXdhbiBLcmVzbm8iLCJmYW1pbHkiOiJEaXBvam9ubyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTcmkgSGFyamF0aSIsImZhbWlseSI6IlN1aGFyZGkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTnVydWwgVGF1ZmlxdSIsImZhbWlseSI6IlJvY2htYW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiRGVkZHkiLCJmYW1pbHkiOiJLdXJuaWFkaSJ9fV0sImlkcyI6eyJpc3NuIjpbIjIyOTYtNjY1WCJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMzM4OS9mZW52cy4yMDIzLjExNTQ3MzkiLCJvcGVuYWxleCI6Ilc0MzI4MTE0OTIzIn0sInRpdGxlIjoiQ29tYmluYXRpb24gb2Ygb3pvbmUtYmFzZWQgYWR2YW5jZWQgb3hpZGF0aW9uIHByb2Nlc3MgYW5kIG5hbm9idWJibGVzIGdlbmVyYXRpb24gdG93YXJkIHRleHRpbGUgd2FzdGV3YXRlciByZWNvdmVyeSIsImFic3RyYWN0IjoiVGhlIGludHJpY2F0ZSBuYXR1cmUgb2YgdmFyaW91cyB0ZXh0aWxlIG1hbnVmYWN0dXJpbmcgcHJvY2Vzc2VzIGludHJvZHVjZXMgY29sb3JlZCBkeWVzLCBzdXJmYWN0YW50cywgYW5kIHRveGljIGNoZW1pY2FscyB0aGF0IGhhdmUgYmVlbiBoYXJtZnVsIHRvIGVjb3N5c3RlbXMgaW4gcmVjZW50IHllYXJzLiBIZXJlLCBhIGNvbWJpbmF0aW9uIG96b25lLWJhc2VkIGFkdmFuY2VkIG94aWRhdGlvbiBwcm9jZXNzIChBT1ApIGlzIGNvdXBsZWQgd2l0aCBhIG5hbm9idWJibGVzIGdlbmVyYXRvciBmb3IgdGhlIGdlbmVyYXRpb24gb2Ygb3pvbmUgbmFub2J1YmJsZXMgKE5CKSB1dGlsaXplZCB0aGUgc2FtZSB0byB0cmVhdCB0aGUgcHJpbWFyeSBlZmZsdWVudCBhY3F1aXJlZCBmcm9tIHRleHRpbGUgd2FzdGV3YXRlcnMuIEhlcmUgd2UgZmluZCBzZXZlcmFsIGtleSBwYXJhbWV0ZXJzIHN1Y2ggYXMgY2hlbWljYWwgb3h5Z2VuIGRlbWFuZCBhbW1vbmlhIGNvbnRlbnQgKE5IIDMgKSwgYW5kIHRvdGFsIHN1c3BlbmRlZCBzb2xpZHMgaW5kaWNhdGluZyBhIHN1YnN0YW50aWFsIHJlY292ZXJ5IGluIHdoaWNoIHRoZSByZXNwZWN0aXZlIHBlcmNlbnRhZ2VzIG9mIDgxLjElLCAzMC44MSUsIGFuZCA0MS45OCUsIHVwb24gMzAwIG1pbiByZXNpZGVuY2UgdGltZSBhcmUgYWNoaWV2ZWQuIE9uIHRoZSBvdGhlciBoYW5kLCB0aGUgcEggaXMgc2hpZnRlZCBmcm9tIDcuOTMgdG8gNy40NiwgaW5kaWNhdGluZyB0aGUgZ2VuZXJhdGlvbiBvZiBoeWRyb2dlbiBwZXJveGlkZSAoSCAyIE8gMiApIGR1ZSB0byB0aGUgdGVybWluYXRpb24gcmVhY3Rpb24gYW5kIHRoZSBzZWxmLXJlYWN0aW9uIG9mIHJlYWN0aXZlIG94eWdlbiBzcGVjaWVzIChST1MpLiBXZSBwcm9wb3NlIHRoYXQgdGhlIHJlYWN0aXZlIG94eWdlbiBzcGVjaWVzIGNhbiBiZSBpZGVudGlmaWVkIGZyb20gdGhlIG5lZ2F0aXZlIHpldGEgcG90ZW50aWFsIG1lYXN1cmVtZW50ICjiiJIyMi40MyDCsSAwLjM0IG1WKSBjb2xsZWN0ZWQgaW4gdGhlIGZpbmFsIHN0YXRlIG9mIHRyZWF0bWVudC4gVGhlIGNvbWJpbmVkIG1ldGhvZCBoYXMgc3VjY2Vzc2Z1bGx5IGdlbmVyYXRlZCBvem9uZSBuYW5vYnViYmxlcyB3aXRoIDk5Ljk0JSBvZiBzaXplIGRpc3RyaWJ1dGVkIGluIDIxNi45IG5tLiBUaGlzIGhpZ2hsaWdodHMgdGhhdCBlbmhhbmNlbWVudCBvZiBvem9uZeKAmXMgcmVhY3Rpdml0eSBwbGF5cyBhIGNydWNpYWwgcm9sZSBpbiBpbXByb3ZpbmcgdGhlIHdhdGVyIHF1YWxpdHkgb2YgdGV4dGlsZSB3YXN0ZXdhdGVyIHRvd2FyZHMgYmVpbmcgdGVjaG5vbG9naWNhbGx5IGVmZmljaWVudCB0byBkYXRlLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzg5L2ZlbnZzLjIwMjMuMTE1NDczOSIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cuZnJvbnRpZXJzaW4ub3JnL2FydGljbGVzLzEwLjMzODkvZmVudnMuMjAyMy4xMTU0NzM5L3BkZiIsImlzc3VlZERhdGUiOiIyMDIzLTAzLTIxIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiRnJvbnRpZXJzIGluIEVudmlyb25tZW50YWwgU2NpZW5jZSIsInZvbHVtZSI6MTEsImltcGFjdEZhY3RvciI6IjUuNjQifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRnJvbnRpZXJzIE1lZGlhIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo2NCwiY2l0ZWRCeUNvdW50IjoxOX19LCJxdW90ZSI6Ik91ciBmaW5kaW5nIHJlZ2FyZGluZyBDT0QgcmVkdWN0aW9uIHRoYXQgb3BlcmF0ZWQgYXQgbmV1dHJhbCBwSCBpbiBhZ3JlZW1lbnQgd2l0aCB0aGUgcHJldmlvdXMgcmVwb3J0cyAoTHVjYXMgZXQgYWwuLCAyMDA5OyBUYWthaGFzaGkgZXQgYWwuLCAyMDEyOyBZYW5nIGV0IGFsLiwgMjAxMjsgVGVtZXNnZW4gZXQgYWwuLCAyMDE3KS4gVGhlIHNlY29uZCByZWNvdmVyeSBzdGFnZSBjb3JyZXNwb25kcyB0byB0aGUgdXRpbGl6YXRpb24gb2Ygb3pvbmUtYmFzZWQgQU9QIGFzIGluZGljYXRlZCBieSB0aGUgYmx1ZSBhcnJvdyBpbiBGaWd1cmUgNy4gSW4gdGhpcyBzZWN0aW9uLCB0aGUgZ2VuZXJhdGlvbiBvZiBST1Mgb3IgbmVnYXRpdmVseS1jaGFyZ2VkIE5CIGRvbWluYW50bHkgY29udHJpYnV0ZWQgdG8gZGVncmFkZSB0aGUgb3JnYW5pYyBjb250YW1pbmFudHMuIFdlIHJldmlzaXQgdGhlIGNvbnZlbnRpb25hbCB0aGVybW9keW5hbWljcyBvZiBidWJibGUgY29sbGFwc2UgcGhlbm9tZW5vbiB0byBkZXNjcmliZSB0aGUgUk9TIGdlbmVyYXRpb24gKEdoYWRpbWtoYW5pIGV0IGFsLiwgMjAxNikgYW5kIG96b25lIGJyZWFrZG93biAoSWtobGFxIGV0IGFsLiwgMjAxMzsgT2ggYW5kIE5ndXllbiwgMjAyMikuIEluIHRoZSBvem9uZSBkZWNvbXBvc2l0aW9uLCB3ZSBhdHRyaWJ1dGUgdGhlIFJPUyBpcyBhIGNoYWluIHJlYWN0aW9uIGNvbXByaXNlZCBvZiBpbml0aWF0aW9uLCBwcm9wYWdhdGlvbiwgYW5kIHRlcm1pbmF0aW9uIChLaHVudGlhIGV0IGFsLiwgMjAxNSkuIEZ1cnRoZXJtb3JlLCBST1MgdmlhIGNvbGxhcHNpbmcgY2F2aXR5IGJ1YmJsZXMgY2FuIGJlIGRlc2NyaWJlZCBiYXNlZCBvbiB0aGUgWW91bmctTGFwbGFjZSBlcXVhdGlvbiAoS2ltdXJhIGV0IGFsLiwgMjAwNDsgVGFrYWhhc2hpIGV0IGFsLiwgMjAwNzsgR2hhZGlta2hhbmkgZXQgYWwuLCAyMDE2KS4gUCDvv70gUDEgKyA0z4Pvv71kICgzKSB3aGVyZSBQIGRlbm90ZXMgZ2FzIHByZXNzdXJlLCBQbCBkZW5vdGVzIGxpcXVpZCBwcmVzc3VyZSwgz4MgZGVub3RlcyBsaXF1aWQgc3VyZmFjZSB0ZW5zaW9uLCBhbmQgZCBkZW5vdGVzIHRoZSBidWJibGUgZGlhbWV0ZXIuIFRoZSBpbnRlcm5hbCBwcmVzc3VyZSBvZiBhIGJ1YmJsZSBpcyBzdWJzdGFudGlhbGx5IGRldGVybWluZWQgYnkgaXRzIHNpemUsIGFjY29yZGluZyB0byBFcS4gMy4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo1OS4yNzg4OTI1MTcwODk4NDQsImxlZnQiOjguMzMzMTcyNzk4MTU2NzM4LCJ3aWR0aCI6NDAuNDcxODM2MDkwMDg3ODksImhlaWdodCI6MS4xNzU3ODM1MTQ5NzY1MDE1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NjAuNjk5NzM3NTQ4ODI4MTI1LCJsZWZ0Ijo4LjMzMzE3MDg5MDgwODEwNSwid2lkdGgiOjQwLjQ3MDkzMjAwNjgzNTk0LCJoZWlnaHQiOjEuMTc1NzgzNTE0OTc2NTAxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjYyLjEyMDU3NDk1MTE3MTg3NSwibGVmdCI6OC4zMzMxNzA4OTA4MDgxMDUsIndpZHRoIjo0MC40NzEzODIxNDExMTMyOCwiaGVpZ2h0IjoxLjE3NTc4MzUxNDk3NjUwMTV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo2My41MzQ3MTc1NTk4MTQ0NSwibGVmdCI6OC4zMzMxNzI3OTgxNTY3MzgsIndpZHRoIjoxNC4xMDIxOTY2OTM0MjA0MSwiaGVpZ2h0IjoxLjE3NTc4MzUxNDk3NjUwMTV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo2NC45NTU1NTg3NzY4NTU0NywibGVmdCI6MTAuNjc1OTg0MzgyNjI5Mzk1LCJ3aWR0aCI6MzguMTMyOTQyMTk5NzA3MDMsImhlaWdodCI6MS4xNzU3ODM1MTQ5NzY1MDE1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NjYuMzc2NDAzODA4NTkzNzUsImxlZnQiOjguMzMzMTc1NjU5MTc5Njg4LCJ3aWR0aCI6NDAuNDcwOTMyMDA2ODM1OTQsImhlaWdodCI6MS4xNzU3ODM1MTQ5NzY1MDE1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NjcuNzk3MjQ4ODQwMzMyMDMsImxlZnQiOjguMzMzMTc3NTY2NTI4MzIsIndpZHRoIjo0LjU0NDMzMjk4MTEwOTYxOSwiaGVpZ2h0IjoxLjE3NTc4MzUxNDk3NjUwMTV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo2Ny43OTcyNDg4NDAzMzIwMywibGVmdCI6MTQuMTkwMjAxNzU5MzM4Mzc5LCJ3aWR0aCI6MS44ODEyOTc3MDc1NTc2NzgyLCJoZWlnaHQiOjEuMTc1NzgzNTE0OTc2NTAxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjY3Ljc5NzI0ODg0MDMzMjAzLCJsZWZ0IjoxNy4zNzEwODYxMjA2MDU0Nywid2lkdGgiOjYuMzc4OTM4MTk4MDg5NiwiaGVpZ2h0IjoxLjE3NTc4MzUxNDk3NjUwMTV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo2Ny43OTcyNDg4NDAzMzIwMywibGVmdCI6MjUuMDY2MDI2Njg3NjIyMDcsIndpZHRoIjoxLjE5MTQzODMxNzI5ODg4OTIsImhlaWdodCI6MS4xNzU3ODM1MTQ5NzY1MDE1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NjcuNzk3MjQ4ODQwMzMyMDMsImxlZnQiOjI3LjU2MTI3MTY2NzQ4MDQ3LCJ3aWR0aCI6Mi43NDI4NjgxODUwNDMzMzUsImhlaWdodCI6MS4xNzU3ODM1MTQ5NzY1MDE1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NjcuNzk3MjQ4ODQwMzMyMDMsImxlZnQiOjMxLjYwODg0NjY2NDQyODcxLCJ3aWR0aCI6MS4zMDEzOTQyMjQxNjY4NzAxLCJoZWlnaHQiOjEuMTc1NzgzNTE0OTc2NTAxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjY3Ljc5NzI0ODg0MDMzMjAzLCJsZWZ0IjozNC4yMTg0MTQzMDY2NDA2MjUsIndpZHRoIjoxMS4yODQ3NzE5MTkyNTA0ODgsImhlaWdodCI6MS4xNzU3ODM1MTQ5NzY1MDE1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NjcuNzk3MjQ4ODQwMzMyMDMsImxlZnQiOjQ2LjgxNzk4NTUzNDY2Nzk3LCJ3aWR0aCI6MS45OTEyNTM0OTUyMTYzNjk2LCJoZWlnaHQiOjEuMTc1NzgzNTE0OTc2NTAxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjY5LjIxODA5Mzg3MjA3MDMxLCJsZWZ0Ijo4LjMzMzE3NzU2NjUyODMyLCJ3aWR0aCI6NDAuNDczNzkzMDI5Nzg1MTU2LCJoZWlnaHQiOjEuMTc1NzgzNTE0OTc2NTAxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjcwLjYzODkzODkwMzgwODYsImxlZnQiOjguMzMzMTc3NTY2NTI4MzIsIndpZHRoIjo0MC40NzE5ODg2Nzc5Nzg1MTYsImhlaWdodCI6MS4xNzU3ODM1MTQ5NzY1MDE1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NzIuMDU5Nzc2MzA2MTUyMzQsImxlZnQiOjguMzMzMTc3NTY2NTI4MzIsIndpZHRoIjo0MC40NjY0MTE1OTA1NzYxNywiaGVpZ2h0IjoxLjE3NTc4MzUxNDk3NjUwMTV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo3My40ODA2MjEzMzc4OTA2MiwibGVmdCI6OC4zMzMxNzc1NjY1MjgzMiwid2lkdGgiOjQwLjQ3NTU5NzM4MTU5MTgsImhlaWdodCI6MS4xNzU3ODM1MTQ5NzY1MDE1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NzQuODk0NzUyNTAyNDQxNCwibGVmdCI6OC4zMzMxNzc1NjY1MjgzMiwid2lkdGgiOjQwLjQ3MjI5Mzg1Mzc1OTc2NiwiaGVpZ2h0IjoxLjE3NTc4MzUxNDk3NjUwMTV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo3Ni4zMTU1OTc1MzQxNzk2OSwibGVmdCI6OC4zMzMxNzg1MjAyMDI2MzcsIndpZHRoIjowLjY1ODIyODA5OTM0NjE2MDksImhlaWdodCI6MS4xNzU3ODM1MTQ5NzY1MDE1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NzYuMzE1NTk3NTM0MTc5NjksImxlZnQiOjEwLjE5OTg2NTM0MTE4NjUyMywid2lkdGgiOjE2Ljg1NTE1NTk0NDgyNDIyLCJoZWlnaHQiOjEuMTc1NzgzNTE0OTc2NTAxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjc2LjMxNTU5NzUzNDE3OTY5LCJsZWZ0IjoyOC4yNjYwMzg4OTQ2NTMzMiwid2lkdGgiOjEuMTkxNDM4MzE3Mjk4ODg5MiwiaGVpZ2h0IjoxLjE3NTc4MzUxNDk3NjUwMTV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo3Ni4zMTU1OTc1MzQxNzk2OSwibGVmdCI6MzAuNjY1OTM3NDIzNzA2MDU1LCJ3aWR0aCI6NS44NDU3Mjc5MjA1MzIyMjcsImhlaWdodCI6MS4xNzU3ODM1MTQ5NzY1MDE1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NzYuMzE1NTk3NTM0MTc5NjksImxlZnQiOjM3LjcyMjgzOTM1NTQ2ODc1LCJ3aWR0aCI6Ny42MTcwNjk3MjEyMjE5MjQsImhlaWdodCI6MS4xNzU3ODM1MTQ5NzY1MDE1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NzYuMzE1NTk3NTM0MTc5NjksImxlZnQiOjQ2LjU1MTA3ODc5NjM4NjcyLCJ3aWR0aCI6Mi4yNTYzNTEyMzI1Mjg2ODY1LCJoZWlnaHQiOjEuMTc1NzgzNTE0OTc2NTAxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjc3LjczNjQ1MDE5NTMxMjUsImxlZnQiOjguMzMzMTc4NTIwMjAyNjM3LCJ3aWR0aCI6MjEuMDgzMDI2ODg1OTg2MzI4LCJoZWlnaHQiOjEuMTc1NzgzNTE0OTc2NTAxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjc5LjE1NzI5NTIyNzA1MDc4LCJsZWZ0IjoxMC42NzU5ODcyNDM2NTIzNDQsIndpZHRoIjozOC4xMjk5MjA5NTk0NzI2NTYsImhlaWdodCI6MS4xNzU3ODM1MTQ5NzY1MDE1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6ODAuNTc4MTQwMjU4Nzg5MDYsImxlZnQiOjguMzMzMDI3ODM5NjYwNjQ1LCJ3aWR0aCI6NDAuNDc3ODU1NjgyMzczMDUsImhlaWdodCI6MS4xNzU3ODM1MTQ5NzY1MDE1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6ODEuOTk4OTc3NjYxMTMyODEsImxlZnQiOjguMzMzMDI2ODg1OTg2MzI4LCJ3aWR0aCI6MzguMTQ3NTQ0ODYwODM5ODQ0LCJoZWlnaHQiOjEuMTc1NzgzNTE0OTc2NTAxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjgzLjEwMTM0MTI0NzU1ODYsImxlZnQiOjI0LjIwOTI3MDQ3NzI5NDkyMiwid2lkdGgiOjI0LjYwMjU5MjQ2ODI2MTcyLCJoZWlnaHQiOjUuMzQ2NDA2NDU5ODA4MzV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo4Ni4xODEzODEyMjU1ODU5NCwibGVmdCI6OC4zMzMyNTQ4MTQxNDc5NSwid2lkdGgiOjQwLjQ3NDQxNDgyNTQzOTQ1LCJoZWlnaHQiOjEuMzU3NjA3OTYwNzAwOTg4OH19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjg3LjYwMjIyNjI1NzMyNDIyLCJsZWZ0Ijo4LjMzMzI3NzcwMjMzMTU0Mywid2lkdGgiOjQwLjQ3NDU0NDUyNTE0NjQ4NCwiaGVpZ2h0IjoxLjE3NTc4MzUxNDk3NjUwMTV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo4OS4wMjMwNzg5MTg0NTcwMywibGVmdCI6OC4zMzM0MzAyOTAyMjIxNjgsIndpZHRoIjo0MC40NzI3NDAxNzMzMzk4NDQsImhlaWdodCI6MS4xNzU3ODM1MTQ5NzY1MDE1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6OTAuNDQzOTIzOTUwMTk1MzEsImxlZnQiOjguMzMzNDMwMjkwMjIyMTY4LCJ3aWR0aCI6NDAuNDc2ODA2NjQwNjI1LCJoZWlnaHQiOjEuMTc1NzgzNTE0OTc2NTAxNX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjYifX1d. These species oxidize recalcitrant organics, ammonia, and nitrites, disrupting molecular bonds without chemical additives. In lab-scale shrimp trials, oxygen nanobubbles reduced Vibrio loads indirectly via ROS-mediated disinfection while lowering TAN, highlighting dual benefits for water chemistry and biosecurity (Nghĩa et al., 2022)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5ndXnhu4VuIEjhu691IiwiZmFtaWx5IjoiTmdoxKlhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5ndXllbiBUaGkiLCJmYW1pbHkiOiJOZ3V5ZW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUGhhbiBUcuG7jW5nIiwiZmFtaWx5IjoiQsOsbmgifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTGUgVGhpIiwiZmFtaWx5IjoiTWF5In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRvbmcgVHJhbiIsImZhbWlseSI6Ikh1eSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQaOG6oW0gVGjDoWkiLCJmYW1pbHkiOiJHaWFuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTb3BoaWUiLCJmYW1pbHkiOiJTdOKAkEhpbGFpcmUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUGhhbiBUaGkiLCJmYW1pbHkiOiJWYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMjM0LTE3NDkiLCIyMjM0LTE3NTciXSwiaXNibiI6W10sImRvaSI6IjEwLjQ3ODUzL2Zhcy4yMDIyLmUzOSIsIm9wZW5hbGV4IjoiVzQyOTQ1OTc1NTQifSwidGl0bGUiOiJFZmZlY3Qgb2YgbmFub2J1YmJsZXMgKG94eWdlbiwgb3pvbmUpIG9uIHRoZSBQYWNpZmljIHdoaXRlIHNocmltcCAoUGVuYWV1cyB2YW5uYW1laSksIFZpYnJpbyBwYXJhaGFlbW9seXRpY3VzIGFuZCB3YXRlciBxdWFsaXR5IHVuZGVyIGxhYiBjb25kaXRpb25zIiwiYWJzdHJhY3QiOiJUaGlzIHN0dWR5IGFzc2Vzc2VkIHRoZSBlZmZlY3RzIG9mIG94eWdlbiBhbmQgb3pvbmUgbmFub2J1YmJsZXMgb24gZ2lsbCBtb3JwaG9sb2d5LCB3ZWlnaHQgZ2FpbiwgYW5kIG1vcnRhbGl0eSBvZiBQYWNpZmljIHdoaXRlIHNocmltcCAoUGVuYWV1cyB2YW5uYW1laSksIGFzIHdlbGwgYXMgdGhlIGxldmVsIG9mIFZpYnJpbyBwYXJhaGFlbW9seXRpY3VzIGFuZCB3YXRlciBxdWFsaXR5IG9mIHNocmltcCBjdWx0dXJlIHRhbmtzIHVuZGVyIGxhYiBjb25kaXRpb25zLiBUd28gZXhwZXJpbWVudHMgd2VyZSBjYXJyaWVkIG91dCB3aXRoIG94eWdlbiBtYWNyb2J1YmJsZSwgb3pvbmUgbWFjcm9idWJibGUsIG94eWdlbiBuYW5vYnViYmxlLCBvem9uZSBuYW5vYnViYmxlLCBhbmQgY29udHJvbCB0cmVhdG1lbnRzIChhaXItc3RvbmUgbWFjcm9idWJibGUpLiBFeHBlcmltZW50cyB3ZXJlIGRvbmUgaW4gdHJpcGxpY2F0ZSBpbiAxMDAgTCB0YW5rcyB3aXRoIDE14oCwIHNhbGluZSB3YXRlciwgYW5kIDIwIHNocmltcCBwZXIgdGFuay4gVGFua3MgaW4gRXhwZXJpbWVudCAxIHdlcmUgbm90IGlub2N1bGF0ZWQgd2l0aCBiYWN0ZXJpYTsgdGFua3MgaW4gRXhwZXJpbWVudCAyIHdlcmUgaW5vY3VsYXRlZCB3aXRoIFYuIHBhcmFoYWVtb2x5dGljdXMgYXQgYSBjb25jZW50cmF0aW9uIG9mIDEwNiBDRlUvbUwuIFRoZSByZXN1bHRzIHJldmVhbGVkIHRoYXQgc2hvcnQgdHJlYXRtZW50cyB3aXRoIG96b25lIG5hbm9idWJibGVzIGhhZCBtaW5pbWFsIGltcGFjdCBvbiBzaHJpbXAgZ2lsbHMsIG1vcnRhbGl0eSwgYW5kIGdyb3d0aCByYXRlcywgcmVkdWNlZCBWLiBwYXJhaGFlbW9seXRpY3VzIGNvbmNlbnRyYXRpb24gaW4gd2F0ZXIgY29tcGFyZWQgdG8gdGhlIG90aGVyIGdyb3VwcywgYW5kIGltcHJvdmUgd2F0ZXIgcXVhbGl0eS4gVGhlc2UgbGFib3JhdG9yeSByZXN1bHRzIGluZGljYXRlIHRoYXQgb3pvbmUgbmFub2J1YmJsZSB0cmVhdG1lbnQgbWF5IGJlIHVzZWZ1bCBmb3IgY29udHJvbGxpbmcgVi4gcGFyYWhhZW1vbHl0aWN1cy4gTW9yZSB3b3JrIGlzIG5lZWRlZCB0byBmaW5kIHRoZSBiZXN0IHByb3RvY29sIHRvIGFwcGx5IHRoZSB0ZWNobm9sb2d5IG9uIGEgY29tbWVyY2lhbCBzY2FsZS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuNDc4NTMvZmFzLjIwMjIuZTM5IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5lLWZhcy5vcmcvZG93bmxvYWQvZG93bmxvYWRfcGRmP3BpZD1mYXMtMjUtOC00MjkiLCJpc3N1ZWREYXRlIjoiMjAyMi0wOC0wMSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZpc2hlcmllcyBhbmQgYXF1YXRpYyBzY2llbmNlcyIsImlzc3VlIjo4LCJ2b2x1bWUiOjI1LCJpbXBhY3RGYWN0b3IiOiIxLjM0In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6NDI5LCJsYXN0UGFnZSI6NDQwfX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlNwcmluZ2VyIFNjaWVuY2UrQnVzaW5lc3MgTWVkaWEifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjMwLCJjaXRlZEJ5Q291bnQiOjEzfX0sInF1b3RlIjoiVGhlIGluZHVzdHJ5IHRoZXJlZm9yZSBuZWVkcyB0byBmaW5kIGFsdGVybmF0aXZlIG1lYXN1cmVzICB0byBwcmV2ZW50IGFuZCBjb250cm9sIHRoaXMgcGF0aG9nZW4uIE5hbm9idWJibGUgdGVjaG5vbG9neSBwcm9kdWNlcyBzbWFsbCBnYXMgYnViYmxlcyB3aXRoIGEgZGlhbWV0ZXIgb2YgbGVzcyB0aGFuIDIwMCBubSAoQWdyYXdhbCBldCBhbC4sIDIwMTE7IENoYXBsaW4sIDIwMjEpIHdoaWNoIGNhbiBleGlzdCBpbiB0aGUgd2F0ZXIgY29sdW1uIGZvciB3ZWVrcyAoQXpldmVkbyAgZXQgYWwuLCAyMDE2OyBQYXJtYXIgJiBNYWp1bWRlciwgMjAxMykuIFRoaXMgcGhlbm9tZW5vbiBpcyAgYXNzb2NpYXRlZCB3aXRoIHRoZSBwaHlzaWNhbCBjaGFyYWN0ZXJpc3RpYyBvZiB0aGVzZSBzbWFsbCBidWItYmxlcyAoV2VpanMgJiBMb2hzZSwgMjAxMykgYW5kIGhpZ2ggemV0YSBwb3RlbnRpYWwgdmFsdWUgd2hpY2ggIGtlZXBzIHRoZW0gZnJvbSBhZ2dyZWdhdGluZyBpbnRvIGxhcmdlciBidWJibGVzIChLaXJieSwgMjAxMDsgIFNqb2dyZWVuIGV0IGFsLiwgMjAxOCkuIE5hbm9idWJibGUgdGVjaG5vbG9neSBoYXMgYSBsb3Qgb2YgcHJvbS1pc2luZyBlZmZlY3RpdmUgYXBwbGljYXRpb24gaW4gYXF1YWN1bHR1cmUgYmVjYXVzZSBvZiBpdHMgYWJpbGl0eSAgdG8gaW5jcmVhc2UgYW5kIG1haW50YWluIG94eWdlbiBjb250ZW50IGluIHdhdGVyIGFuZCBjb250cm9sICBiYWN0ZXJpYSwgYSBwb3NzaWJsZSBtZWFzdXJlIHRvIHJlZHVjZSB0aGUgdXNlIG9mIGFudGliaW90aWNzLiBPeHlnZW4gbmFub2J1YmJsZXMgb3B0aW1pemUgdGhlIGRpc3NvbHZlZCBveHlnZW4gaW4gYXF1YXRpYyBzeXN0ZW1zIChHYWxhbmcgZXQgYWwuLCAyMDE5KS4gSGlnaCBsZXZlbHMgb2Ygb3h5Z2VuIG1heSBzcGVlZCB1cCBtZXRhYm9saXNtLCBwcm9tb3RlIGhpZ2hlciBmb29kIGludGFrZSwgYW5kIGluY3JlYXNlICBncm93dGggb2YgZmFybWVkIHNocmltcCAoRWJpbmEgZXQgYWwuLCAyMDEzKS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNS4wNjg2NDU0NzcyOTQ5MiwibGVmdCI6OC4wOTUyMzU4MjQ1ODQ5NjEsIndpZHRoIjo0MS4wNzIyMTIyMTkyMzgyOCwiaGVpZ2h0IjoxLjY5OTYzNTE0ODA0ODQwMDl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNi44MzI1MzA5NzUzNDE4LCJsZWZ0Ijo4LjA5NTIzNTgyNDU4NDk2MSwid2lkdGgiOjIzLjYwNDE4NTEwNDM3MDExNywiaGVpZ2h0IjoxLjY5OTYzNTE0ODA0ODQwMDl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC41OTY0MjAyODgwODU5NCwibGVmdCI6MTAuOTUyMzY0OTIxNTY5ODI0LCJ3aWR0aCI6MzguMjE1ODc3NTMyOTU4OTg0LCJoZWlnaHQiOjEuNjk5NjM1MTQ4MDQ4NDAwOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQwLjM2MDMwOTYwMDgzMDA4LCJsZWZ0Ijo4LjA5NTIzNTgyNDU4NDk2MSwid2lkdGgiOjQxLjA3Njc1OTMzODM3ODkwNiwiaGVpZ2h0IjoxNy41NzQ2MzA3MzczMDQ2ODh9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1Ny45OTkxOTUwOTg4NzY5NSwibGVmdCI6MTAuOTUyMzY0OTIxNTY5ODI0LCJ3aWR0aCI6MzguMjAxNDQ2NTMzMjAzMTI1LCJoZWlnaHQiOjEuNjk5NjM1MTQ4MDQ4NDAwOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjU5Ljc2MzA4MDU5NjkyMzgzLCJsZWZ0Ijo4LjA5NTIzNTgyNDU4NDk2MSwid2lkdGgiOjQxLjA2MzMyNzc4OTMwNjY0LCJoZWlnaHQiOjEuNjk5NjM1MTQ4MDQ4NDAwOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjYxLjUyNjk3MzcyNDM2NTIzNCwibGVmdCI6OC4wOTUyMzU4MjQ1ODQ5NjEsIndpZHRoIjo0MS4wNjY4NDQ5NDAxODU1NSwiaGVpZ2h0IjoxLjY5OTYzNTE0ODA0ODQwMDl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2My4yOTA4NjMwMzcxMDkzNzUsImxlZnQiOjguMDk1MjM1ODI0NTg0OTYxLCJ3aWR0aCI6NDAuNzE0MjkwNjE4ODk2NDg0LCJoZWlnaHQiOjEuNjk5NjMxMzMzMzUxMTM1M319XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjQzMCJ9fV0%3D. Surface-active properties, including negative zeta potentials (-20 to -50 mV), prevent coalescence and enhance pollutant adsorption via hydrophobic interactions (Jia et al., 2023; Wu et al., 2021)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjMwLCJzdWJ0eXBlIjp7fSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTWluZ3lpIiwiZmFtaWx5IjoiSmlhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik11aGFtbWFkIFVzbWFuIiwiZmFtaWx5IjoiRmFyaWQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSmVoYWQgQS4iLCJmYW1pbHkiOiJLaGFycmF6In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hbGxhcGFuZW5pIE1hbm9qIiwiZmFtaWx5IjoiS3VtYXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2hhdWhyYXQgUy4iLCJmYW1pbHkiOiJDaG9wcmEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW0iLCJmYW1pbHkiOiJKYW5nIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlkuTS4gSm9obiIsImZhbWlseSI6IkNoZXcifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2FtaXIgS3VtYXIiLCJmYW1pbHkiOiJLaGFuYWwifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiR3VhbmdoYW8iLCJmYW1pbHkiOiJDaGVuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFsaWNpYSBLeW91bmdqaW4iLCJmYW1pbHkiOiJBbiJ9fV0sImlkcyI6eyJpc3NuIjpbIjAwNDMtMTM1NCIsIjE4NzktMjQ0OCJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMTAxNi9qLndhdHJlcy4yMDIzLjEyMDYxMyIsInBtaWQiOiIzNzczODk0MCIsIm9wZW5hbGV4IjoiVzQzODY2MTg0NTkifSwidGl0bGUiOiJOYW5vYnViYmxlcyBpbiB3YXRlciBhbmQgd2FzdGV3YXRlciB0cmVhdG1lbnQgc3lzdGVtczogU21hbGwgYnViYmxlcyBtYWtpbmcgYmlnIGRpZmZlcmVuY2UiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAxNi9qLndhdHJlcy4yMDIzLjEyMDYxMyIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9wdXJlaG9zdC5iYXRoLmFjLnVrL3dzL2ZpbGVzLzI5NjM0MzkwMS9OYW5vYnViYmxlc19wcmVfcHJvb2YucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjMtMDktMTEiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA4IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJXYXRlciBSZXNlYXJjaCIsInZvbHVtZSI6MjQ1LCJpbXBhY3RGYWN0b3IiOiIxMy40NCJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjEyMDYxMywibGFzdFBhZ2UiOjEyMDYxM319LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJFbHNldmllciBCViJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MTExLCJjaXRlZEJ5Q291bnQiOjc1fX0sInF1b3RlIjoiU2V2ZXJhbCBzdHVkaWVzIGhhdmUgcHJvcG9zZWQgdGhhdCB0aGUgbmVnYXRpdmUgY2hhcmdlIG9mIE5CcyBhcmlzZXMgZHVlIHRvIHRoZWlyIHNtYWxsIHJhZGl1cyBhbmQgdGhlIGhpZ2ggcG9sYXJpdHkgb2YgaHlkcm94eWwgaW9ucywgd2hpY2ggbGVhZCB0byB0aGUgYWRzb3JwdGlvbiBvZiBkaXNzb2NpYXRlZCBoeWRyb3h5bCBpb25zIG9uIHRoZSBidWJibGUgc3VyZmFjZSwgd2hpbGUgaHlkcm9nZW4gaW9ucyBhcmUgZWFzaWx5IGh5ZHJhdGVkIGFuZCBwcmVmZXIgdG8gc3RheSBpbiB0aGUgYnVsayBsaXF1aWQuIEEgaGlnaGVyIGFic29sdXRlIHpldGEgcG90ZW50aWFsIGluZGljYXRlcyBoaWdoZXIgc3RhYmlsaXR5LCBhcyB0aGUgcmVwdWxzaXZlIGZvcmNlIGdlbmVyYXRlZCBieSBOQnPigJggc3VyZmFjZXMgcHJldmVudHMgdGhlbSBmcm9tIG1lcmdpbmcgd2l0aCBvbmUgYW5vdGhlciAoVXNoaWt1Ym8gZXQgYWwuLCAyMDEwKS4gMTEgU3VyZmFjZSBoeWRyb3Bob2JpY2l0eSBpcyBhbm90aGVyIGVzc2VudGlhbCBwcm9wZXJ0eSBvZiBOQnMgYXMgaXQgYWxsb3dzIE5CcyB0byBpbnRlcmFjdCB3aXRoIG90aGVyIHBhcnRpY2xlcyB0aHJvdWdoIGh5ZHJvcGhvYmljIGF0dHJhY3Rpb24uIEJOQnMgY2FuIGFkc29yYiBvbiBoeWRyb3Bob2JpYyBwYXJ0aWNsZXMgYW5kIGluY3JlYXNlIHRoZWlyIG1vYmlsaXR5IGluIHRoZSBsaXF1aWQsIHdoaWxlIGF0dGFjaGVkIE5CcyBjYW4gcHJvbW90ZSBwYXJ0aWNsZS1wYXJ0aWNsZSBvciBwYXJ0aWNsZS0gYnViYmxlIGludGVyYWN0aW9ucyB0aHJvdWdoIGNhcGlsbGFyeSBicmlkZ2luZyBlZmZlY3RzIHRoYXQgcHJvZHVjZSBhbiBhdHRyYWN0aXZlIGZvcmNlIGJldHdlZW4gdHdvIHN1cmZhY2VzIChIYW1wdG9uIGFuZCBOZ3V5ZW4sIDIwMTApLiAyLjEuMyBMb25nIHJlc2lkZW5jZSB0aW1lIGluIHdhdGVyOiBOQnPigJkgbG9uZy10ZXJtIHN0YWJpbGl0eSBBbW9uZyB0aGUgdmFyaW91cyBjaGFyYWN0ZXJpc3RpY3MgTkJzIHBvc3Nlc3MsIHRoZSBtb3N0IGFzdG9uaXNoaW5nIGlzIHRoZWlyIGxvbmdldml0eS4gVXBvbiBnZW5lcmF0aW9uLCBidWJibGVzIGdlbmVyYWxseSBtYWludGFpbiB0aGVpciBkaWFtZXRlcnMgZm9yIGEgY2VydGFpbiBhbW91bnQgb2YgdGltZSwgYWZ0ZXIgd2hpY2ggdGhlIGJ1YmJsZXMgYmVnaW4gdG8gZ3JvdyBhbmQgZXZlbnR1YWxseSBkaXNzb2x2ZSBpbiB0aGUgc29sdXRpb24uIEV4cGVyaW1lbnRhbCByZXN1bHRzIGhhdmUgY29uZmlybWVkIHRoYXQgTkJzIGFyZSBhYmxlIHRvIGV4aXN0IGluIHdhdGVyIGZyb20gZGF5cyB0byBtb250aHMuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxMSwiYmJveCI6eyJ0b3AiOjY4LjYzMTk2NTYzNzIwNzAzLCJsZWZ0Ijo5Ljc0MTE4MDQxOTkyMTg3NSwid2lkdGgiOjc3LjAxMDI1MzkwNjI1LCJoZWlnaHQiOjEuODQ5MjYzNjY4MDYwMzAyN319LHsicGFnZU51bWJlciI6MTEsImJib3giOnsidG9wIjo3MS44MjM4MjIwMjE0ODQzOCwibGVmdCI6OS43NDExODA0MTk5MjE4NzUsIndpZHRoIjo3Ny4wMDYzMDk1MDkyNzczNCwiaGVpZ2h0IjoxLjg0OTI2MzY2ODA2MDMwMjd9fSx7InBhZ2VOdW1iZXIiOjExLCJiYm94Ijp7InRvcCI6NzUuMDEyMjE0NjYwNjQ0NTMsImxlZnQiOjkuNzQxMTgwNDE5OTIxODc1LCJ3aWR0aCI6NzYuOTQ4Nzc2MjQ1MTE3MTksImhlaWdodCI6MS44NDkyNjM2NjgwNjAzMDI3fX0seyJwYWdlTnVtYmVyIjoxMSwiYmJveCI6eyJ0b3AiOjc4LjIwMDU5OTY3MDQxMDE2LCJsZWZ0Ijo5Ljc0MTE4MDQxOTkyMTg3NSwid2lkdGgiOjc3LjAwODI4NTUyMjQ2MDk0LCJoZWlnaHQiOjEuODQ5MjYzNjY4MDYwMzAyN319LHsicGFnZU51bWJlciI6MTEsImJib3giOnsidG9wIjo4MS4zODg5OTk5Mzg5NjQ4NCwibGVmdCI6OS43NDExODA0MTk5MjE4NzUsIndpZHRoIjo3Ny4wMTAyNDYyNzY4NTU0NywiaGVpZ2h0IjoxLjg0OTI2MzY2ODA2MDMwMjd9fSx7InBhZ2VOdW1iZXIiOjExLCJiYm94Ijp7InRvcCI6ODQuNTc2OTE5NTU1NjY0MDYsImxlZnQiOjkuNzQxMTgwNDE5OTIxODc1LCJ3aWR0aCI6MTkuNjE2NjI0ODMyMTUzMzIsImhlaWdodCI6MS44NDkyNjM2NjgwNjAzMDI3fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjg3LjQ4ODA1OTk5NzU1ODYsImxlZnQiOjQ3LjAxODI1MzMyNjQxNjAxNiwid2lkdGgiOjIuNDUyMTYxNzg4OTQwNDI5NywiaGVpZ2h0IjoxLjg0OTI2MzY2ODA2MDMwMjd9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MTEuNjQ4NzM4ODYxMDgzOTg0LCJsZWZ0Ijo5Ljc0MTE4MDQxOTkyMTg3NSwid2lkdGgiOjc2Ljk1MjcwNTM4MzMwMDc4LCJoZWlnaHQiOjEuODQ5MjY1NTc1NDA4OTM1NX19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoxNC44Mzk0MzQ2MjM3MTgyNjIsImxlZnQiOjkuNzQxMTgwNDE5OTIxODc1LCJ3aWR0aCI6NzYuOTQ4NzgzODc0NTExNzIsImhlaWdodCI6MS44NDkyNjU1NzU0MDg5MzU1fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjE4LjAyNzgzMzkzODU5ODYzMywibGVmdCI6OS43NDExODA0MTk5MjE4NzUsIndpZHRoIjo3Ni41MDYwNzI5OTgwNDY4OCwiaGVpZ2h0IjoxLjg0OTI2NTU3NTQwODkzNTV9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjEuMjE2MjE3MDQxMDE1NjI1LCJsZWZ0Ijo5Ljc0MTE4MDQxOTkyMTg3NSwid2lkdGgiOjc2Ljk4NTczMzAzMjIyNjU2LCJoZWlnaHQiOjEuODQ5MjY1NTc1NDA4OTM1NX19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyNC40MDQ2MDAxNDM0MzI2MTcsImxlZnQiOjkuNzQxMTgwNDE5OTIxODc1LCJ3aWR0aCI6MzUuMzc0MTk4OTEzNTc0MjIsImhlaWdodCI6MS44NDkyNjU1NzU0MDg5MzU1fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjMyLjIxMTMxMTM0MDMzMjAzLCJsZWZ0Ijo5Ljc0MTE4MDQxOTkyMTg3NSwid2lkdGgiOjUyLjQ2NDEyMjc3MjIxNjgsImhlaWdodCI6MS45MTk5NjUyNjcxODEzOTY1fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM2LjA1MjYyNzU2MzQ3NjU2LCJsZWZ0Ijo5Ljc0MTE4MDQxOTkyMTg3NSwid2lkdGgiOjc3LjAwODI4NTUyMjQ2MDk0LCJoZWlnaHQiOjEuODQ5MjY3NDgyNzU3NTY4NH19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozOS4yNDEwMTYzODc5Mzk0NSwibGVmdCI6OS43NDExODA0MTk5MjE4NzUsIndpZHRoIjo3Ny4wMDgyODU1MjI0NjA5NCwiaGVpZ2h0IjoxLjg0OTI2NzQ4Mjc1NzU2ODR9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDIuNDI5NDA5MDI3MDk5NjEsImxlZnQiOjkuNzQxMTgwNDE5OTIxODc1LCJ3aWR0aCI6NzcuMDAyNDAzMjU5Mjc3MzQsImhlaWdodCI6MS44NDkyNjc0ODI3NTc1Njg0fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQ1LjYxNzc5NDAzNjg2NTIzNCwibGVmdCI6OS43NDExODA0MTk5MjE4NzUsIndpZHRoIjo3Ny4wMTAyNDYyNzY4NTU0NywiaGVpZ2h0IjoxLjg0OTI2NzQ4Mjc1NzU2ODR9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIxMjA2MjQifX0seyJvcmlnaW4iOjQsIndvcmsiOnsidHlwZSI6MzAsInN1YnR5cGUiOnt9LCJjb250cmlidXRvcnMiOlt7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJKaWFqaWEiLCJmYW1pbHkiOiJXdSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLZWppYSIsImZhbWlseSI6IlpoYW5nIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkNoZW5nIiwiZmFtaWx5IjoiQ2VuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlhpYW9nYW5nIiwiZmFtaWx5IjoiV3UifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUnV5aW4iLCJmYW1pbHkiOiJNYW8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiWWluZ3lpbmciLCJmYW1pbHkiOiJaaGVuZyJ9fV0sImlkcyI6eyJpc3NuIjpbIjIxOTEtMDg1NSJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMTE4Ni9zMTM1NjgtMDIxLTAxMjU0LTAiLCJtYWciOiIzMTc1NjgzNTYxIiwicG1pZCI6IjM0MTg0MTM3IiwicG1jaWQiOiI4MjM5MTA5Iiwib3BlbmFsZXgiOiJXMzE3NTY4MzU2MSJ9LCJ0aXRsZSI6IlJvbGUgb2YgYnVsayBuYW5vYnViYmxlcyBpbiByZW1vdmluZyBvcmdhbmljIHBvbGx1dGFudHMgaW4gd2FzdGV3YXRlciB0cmVhdG1lbnQiLCJhYnN0cmFjdCI6IkFic3RyYWN0IFRoZSBvY2N1cnJlbmNlIG9mIGEgdmFyaWV0eSBvZiBvcmdhbmljIHBvbGx1dGFudHMgaGFzIGNvbXBsaWNhdGVkIHdhc3Rld2F0ZXIgdHJlYXRtZW50OyB0aHVzLCB0aGUgc2VhcmNoIGZvciBzdXN0YWluYWJsZSBhbmQgZWZmZWN0aXZlIHRyZWF0bWVudCB0ZWNobm9sb2d5IGhhcyBkcmF3biBzaWduaWZpY2FudCBhdHRlbnRpb24uIEluIHJlY2VudCB5ZWFycywgYnVsayBuYW5vYnViYmxlcywgd2hpY2ggaGF2ZSBleHRyYW9yZGluYXJ5IHByb3BlcnRpZXMgZGlmZmVyaW5nIGZyb20gdGhvc2Ugb2YgbWljcm9idWJibGVzLCBpbmNsdWRpbmcgaGlnaCBzdGFiaWxpdHkgYW5kIGxvbmcgcmVzaWRlbmNlIHRpbWVzIGluIHdhdGVyLCBsYXJnZSBzcGVjaWZpYyBzdXJmYWNlIGFyZWFzLCBoaWdoIGdhcyB0cmFuc2ZlciBlZmZpY2llbmN5IGFuZCBpbnRlcmZhY2UgcG90ZW50aWFsLCBhbmQgdGhlIGNhcGFiaWxpdHkgdG8gZ2VuZXJhdGUgZnJlZSByYWRpY2FscywgaGF2ZSBzaG93biBhdHRyYWN0aXZlIHRlY2hub2xvZ2ljYWwgYWR2YW50YWdlcyBhbmQgcHJvbWlzaW5nIGFwcGxpY2F0aW9uIHByb3NwZWN0cyBmb3Igd2FzdGV3YXRlciB0cmVhdG1lbnQuIEluIHRoaXMgcmV2aWV3LCB0aGUgYmFzaWMgY2hhcmFjdGVyaXN0aWNzIG9mIGJ1bGsgbmFub2J1YmJsZXMgYXJlIHN1bW1hcml6ZWQgaW4gZGV0YWlsLCBhbmQgcmVjZW50IGZpbmRpbmdzIHJlbGF0ZWQgdG8gdGhlaXIgaW1wbGVtZW50YXRpb24gcGF0aHdheXMgYW5kIG1lY2hhbmlzbXMgaW4gb3JnYW5pYyB3YXN0ZXdhdGVyIHRyZWF0bWVudCBhcmUgc3lzdGVtYXRpY2FsbHkgZGlzY3Vzc2VkLCB3aGljaCBpbmNsdWRlcyBpbXByb3ZpbmcgdGhlIGFpciBmbG90YXRpb24gcHJvY2VzcywgaW5jcmVhc2luZyB3YXRlciBhZXJhdGlvbiB0byBwcm9tb3RlIGFlcm9iaWMgYmlvbG9naWNhbCB0ZWNobm9sb2dpZXMgaW5jbHVkaW5nIGJpb2xvZ2ljYWwgYWN0aXZhdGVkIGNhcmJvbiwgYWN0aXZhdGVkIHNsdWRnZSwgYW5kIG1lbWJyYW5lIGJpb3JlYWN0b3JzLCBhbmQgZ2VuZXJhdGluZyBhY3RpdmUgZnJlZSByYWRpY2FscyB0aGF0IG94aWRpc2Ugb3JnYW5pYyBjb21wb3VuZHMuIEZpbmFsbHksIHRoZSBjdXJyZW50IHRlY2hub2xvZ2ljYWwgZGlmZmljdWx0aWVzIG9mIGJ1bGsgbmFub2J1YmJsZXMgYXJlIGFuYWx5c2VkLCBhbmQgZnV0dXJlIGZvY3VzIGFyZWFzIGZvciByZXNlYXJjaCBvbiBidWxrIG5hbm9idWJibGUgdGVjaG5vbG9neSBhcmUgYWxzbyBwcm9wb3NlZC4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTE4Ni9zMTM1NjgtMDIxLTAxMjU0LTAiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vYW1iLWV4cHJlc3Muc3ByaW5nZXJvcGVuLmNvbS9jb3VudGVyL3BkZi8xMC4xMTg2L3MxMzU2OC0wMjEtMDEyNTQtMCIsImlzc3VlZERhdGUiOiIyMDIxLTA2LTI4IiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiQU1CIEV4cHJlc3MiLCJpc3N1ZSI6MSwidm9sdW1lIjoxMSwiaW1wYWN0RmFjdG9yIjoiNC4yNiJ9LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJTcHJpbmdlciBOYXR1cmUifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjc2LCJjaXRlZEJ5Q291bnQiOjc1fX0sInF1b3RlIjoiVGhlIG1vZGVsIHN1Y2Nlc3NmdWxseSBlbHVjaWRhdGVzIHJlcHVsc2l2ZSBmb3JjZXMgIGJldHdlZW4gT0jiiJIgb24gdGhlIHN1cmZhY2UgYXJlIHN1ZmZpY2llbnQgdG8gdWx0aS1tYXRlbHkgYmFsYW5jZSB0aGUgc3VyZmFjZSB0ZW5zaW9uIGZvcmNlcyBhdCBuYW5vYnViYmxlICBzaXplcywgdGh1cywgbWFraW5nIG5hbm9idWJibGVzIGV4aGliaXQgZ3JlYXQgc3RhYmlsLWl0eS4gQXMgc2hvd24gaW4gcHJldmlvdXMgbGl0ZXJhdHVyZXMsIHpldGEgcG90ZW50aWFsIHZhbC11ZXMgY2hhbmdlIHdpdGggdGhlIHBIIG9mIHRoZSBtZWRpdW0gdmFyeSwgZXhoaWJpdGluZyAgYSB0ZW5kZW5jeSB0aGF0IHRoZSBuZWdhdGl2aXR5IG9mIHpldGEgcG90ZW50aWFsIGluY3JlYXNlcyAgd2l0aCBzb2x1dGlvbiBwSCBpbmNyZWFzZSB3aGljaCByZXN1bHRzIGluIGdyZWF0ZXIgYnViLWJsZSBzdXJmYWNlIGNoYXJnZSBhbmQgbWFrZSBidWJibGVzIG1vcmUgc3RhYmxlIGR1ZSB0byAgZWxlY3Ryb3N0YXRpYyByZXB1bHNpb24gKE1lZWdvZGEgZXQgYWwuIDIwMTgpLiBOYW5vYnViLWJsZXMgZ2VuZXJhbGx5IHNob3cgaGlnaCBuZWdhdGl2ZSB6ZXRhIHBvdGVudGlhbCBvd2luZyB0byAgdGhlIGV4Y2VzcyBvZiBoeWRyb3hpZGUgaW9ucyAoT0jiiJIpIHJlbGF0aXZlIHRvIGh5ZHJvZ2VuICBpb25zIChIKykgYXQgdGhlIGdhc%2BKAk3dhdGVyIGludGVyZmFjZSBvZiBidWJibGVzIHVuZGVyICB3aWRlIHJhbmdlIG9mIHNvbHV0aW9uIHBILCBidXQgbmFub2J1YmJsZXMgZm9ybWVkIGluICBhY2lkaWMgZW52aXJvbm1lbnRzIGFsd2F5cyBzaG93IGxvd2VyIHN0YWJpbGl0eSBjb20tcGFyZWQgd2l0aCBpbiBuZXV0cmFsIG9yIGFsa2FsaW5lIGNvbmRpdGlvbnMgYmVjYXVzZSBhICBsYWNrIG9mIE9I4oiSIGFyZSByZXF1aXJlZCB0byBzdGFiaWxpemUgdGhlIGVsZWN0cmljIGRvdWJsZSAgbGF5ZXIgKE5pcm1hbGthciBldCBhbC4gMjAxOCksIHRoZSB6ZXRhIHBvdGVudGlhbCBvZiBuYW5vLWJ1YmJsZXMgZ2VuZXJhdGVkIGluIHN0cm9uZyBhY2lkaWMgbWVkaXVtIGJlbG93IHBIIDMgIHRlbmRzIHRvIGJlIHBvc2l0aXZlIGFuZCBsb3cgd2hpY2ggY2F1c2VzIGhpZ2hlciBwb3NzaS1iaWxpdHkgZm9yIGJ1YmJsZSBjb2FsZXNjZW5jZSAoWmhhbmcgZXQgYWwuIDIwMjApLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjEzLjk2ODMwNTU4Nzc2ODU1NSwibGVmdCI6NTIuNTM0MzQ3NTM0MTc5NjksIndpZHRoIjozOC4zMjY5MTU3NDA5NjY4LCJoZWlnaHQiOjEuODM3NjU2NjE3MTY0NjExOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjE1LjQ3NzMzNDAyMjUyMTk3MywibGVmdCI6NTEuMTkwOTcxMzc0NTExNzIsIndpZHRoIjozOS4yODUxNjM4NzkzOTQ1MywiaGVpZ2h0IjoxLjg0NjU2MTU1MTA5NDA1NTJ9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjoxNy4wMDM1NjI5MjcyNDYwOTQsImxlZnQiOjUxLjE5MDQzNzMxNjg5NDUzLCJ3aWR0aCI6MzkuNjY5MTA1NTI5Nzg1MTU2LCJoZWlnaHQiOjEuODM3NjU2NjE3MTY0NjExOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjE4LjUyMDI3NzAyMzMxNTQzLCJsZWZ0Ijo1MS4xOTA0MzczMTY4OTQ1Mywid2lkdGgiOjM5LjI4NTY5NDEyMjMxNDQ1LCJoZWlnaHQiOjEuODM4MjY2MjUzNDcxMzc0NX19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjIwLjAzODIxMTgyMjUwOTc2NiwibGVmdCI6NTEuMTkwNDM3MzE2ODk0NTMsIndpZHRoIjozOS4yODU2OTQxMjIzMTQ0NSwiaGVpZ2h0IjoxLjgzNzY1NjYxNzE2NDYxMTh9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjoyMS41NTU1MzYyNzAxNDE2LCJsZWZ0Ijo1MS4xOTA0MzczMTY4OTQ1Mywid2lkdGgiOjM5LjY3NTIxNjY3NDgwNDY5LCJoZWlnaHQiOjEuODM3NjU2NjE3MTY0NjExOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjIzLjA3MjI0ODQ1ODg2MjMwNSwibGVmdCI6NTEuMTkwNDM3MzE2ODk0NTMsIndpZHRoIjozOS42NzAzMjYyMzI5MTAxNTYsImhlaWdodCI6MS44Mzc2NTY2MTcxNjQ2MTE4fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MjQuNTg4OTYyNTU0OTMxNjQsImxlZnQiOjUxLjE5MDQzNzMxNjg5NDUzLCJ3aWR0aCI6MzkuMjg1Njk0MTIyMzE0NDUsImhlaWdodCI6MS44Mzg4NzU4ODk3NzgxMzcyfX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MjYuMTA3NTA3NzA1Njg4NDc3LCJsZWZ0Ijo1MS4xOTA0MzczMTY4OTQ1Mywid2lkdGgiOjM5LjY3MjI1MjY1NTAyOTMsImhlaWdodCI6MS44Mzc2NTY2MTcxNjQ2MTE4fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MjcuNjI0MjIxODAxNzU3ODEyLCJsZWZ0Ijo1MS4xOTA0MzczMTY4OTQ1Mywid2lkdGgiOjM5LjI4NTY5NDEyMjMxNDQ1LCJoZWlnaHQiOjEuODM4MjY2MjUzNDcxMzc0NX19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjI5LjE0MjE1NjYwMDk1MjE1LCJsZWZ0Ijo1MS4xOTA0MzczMTY4OTQ1Mywid2lkdGgiOjM5LjY3NTIzNTc0ODI5MTAxNiwiaGVpZ2h0IjoxLjgzNzY1NjYxNzE2NDYxMTh9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjozMC42NTA1NzM3MzA0Njg3NSwibGVmdCI6NTEuMTkwNDM3MzE2ODk0NTMsIndpZHRoIjozOS42NzE3MjYyMjY4MDY2NCwiaGVpZ2h0IjoxLjg0NjU1ODgwOTI4MDM5NTV9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjozMi4xNjc5MDAwODU0NDkyMiwibGVmdCI6NTEuMTg5ODk5NDQ0NTgwMDgsIndpZHRoIjozOS42Njk5NDA5NDg0ODYzMywiaGVpZ2h0IjoxLjg0NjU1ODgwOTI4MDM5NTV9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjozMy42OTM1MTU3Nzc1ODc4OSwibGVmdCI6NTEuMTkwODYwNzQ4MjkxMDE2LCJ3aWR0aCI6MzkuNjc1NzIwMjE0ODQzNzUsImhlaWdodCI6MS44Mzc2NTQ1OTA2MDY2ODk1fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MzUuMjEwMjM1NTk1NzAzMTI1LCJsZWZ0Ijo1MS4xOTA4NjA3NDgyOTEwMTYsIndpZHRoIjozOS4yODUyNzA2OTA5MTc5NywiaGVpZ2h0IjoxLjgzODg3NDEwMTYzODc5NH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjM2LjcyODc3ODgzOTExMTMzLCJsZWZ0Ijo1MS4xOTA0MzczMTY4OTQ1Mywid2lkdGgiOjM5LjY2ODk3MjAxNTM4MDg2LCJoZWlnaHQiOjEuODM3NjU0NTkwNjA2Njg5NX19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjM4LjIzNzE3ODgwMjQ5MDIzNCwibGVmdCI6NTEuMTkwNDM3MzE2ODk0NTMsIndpZHRoIjozOS42Njg4MDc5ODMzOTg0NCwiaGVpZ2h0IjoxLjg0NjU4MDAyODUzMzkzNTV9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjozOS43NjI4MjExOTc1MDk3NjYsImxlZnQiOjUxLjE4OTk1Mjg1MDM0MTgsIndpZHRoIjozOS4yODYxODI0MDM1NjQ0NSwiaGVpZ2h0IjoxLjgzODI2MDUzMTQyNTQ3Nn19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjQxLjI4MDc1MDI3NDY1ODIsImxlZnQiOjUxLjE5MDQzNzMxNjg5NDUzLCJ3aWR0aCI6MzkuNjcyNzMzMzA2ODg0NzY2LCJoZWlnaHQiOjEuODM3NjU0NTkwNjA2Njg5NX19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjQyLjc5NzQ3MDA5Mjc3MzQ0LCJsZWZ0Ijo1MS4xOTA0MzczMTY4OTQ1Mywid2lkdGgiOjM5LjI4NTY5NDEyMjMxNDQ1LCJoZWlnaHQiOjEuODM4MjYwNTMxNDI1NDc2fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6NDQuMzE1Mzk5MTY5OTIxODc1LCJsZWZ0Ijo1MS4xOTA0MzczMTY4OTQ1Mywid2lkdGgiOjM5LjI4NTY5NDEyMjMxNDQ1LCJoZWlnaHQiOjEuODM3NjU0NTkwNjA2Njg5NX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjMifX1d.

Scalability and optimization of nanobubble systems are key to practical adoption, with generators modifiable via perforated plate geometry (e.g., hole number and diameter) to boost oxygen concentrations while controlling temperature (Zhu et al., 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5pbmciLCJmYW1pbHkiOiJaaHUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTWluLVl1IiwiZmFtaWx5IjoiTGkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiS29oZWkiLCJmYW1pbHkiOiJTaGliYXRhIn19XSwiaWRzIjp7Imlzc24iOltdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMzM5MC9lbmdwcm9jMjAyMzA1NTAyMyIsIm9wZW5hbGV4IjoiVzQzODkxNDU1NzgiLCJleGEiOiJodHRwczovL3d3dy5tZHBpLmNvbS8yNjczLTQ1OTEvNTUvMS8yMyJ9LCJ0aXRsZSI6IlRoZSBEZXZlbG9wbWVudCBvZiBhIEhpZ2gtQ29uY2VudHJhdGlvbiBPeHlnZW5hdGVkIFdhdGVyIEdlbmVyYXRvciBCYXNlZCBvbiBOYW5vYnViYmxlcyBhbmQgSXRzIEFwcGxpY2F0aW9uIiwiYWJzdHJhY3QiOiJXYXRlciB3aXRoIGEgaGlnaCBjb25jZW50cmF0aW9uIG9mIG94eWdlbiBpcyBuZWVkZWQgZm9yIHRoZSBhcXVhY3VsdHVyZSBpbmR1c3RyeSBpbiBKYXBhbi4gSW4gdGhlIGN1cnJlbnQgc3R1ZHksIHRoZSBwcmVzc3VyaXplZCBkaXNzb2x1dGlvbiBtZXRob2Qgd2FzIGVtcGxveWVkIHRvIGdlbmVyYXRlIGhpZ2gtY29uY2VudHJhdGlvbiBveHlnZW5hdGVkIHdhdGVyIChIQ09XKSBieSBwcm9kdWNpbmcgb3h5Z2VuIG5hbm9idWJibGVzIGluIHRoZSB3YXRlci4gSW4gb3JkZXIgdG8gaW52ZXN0aWdhdGUgZmFjdG9ycyBzdWNoIGFzIHRlbXBlcmF0dXJlLCBnZW9tZXRyaWMgY29uZGl0aW9ucywgYW5kIHRoZWlyIGluZmx1ZW5jZSBvbiB0aGUgb3h5Z2VuIGNvbmNlbnRyYXRpb24sIGEgc3BlY2lhbCBuYW5vYnViYmxlIGdlbmVyYXRvciB3YXMgaW1wcm92ZWQgYnkgY2hhbmdpbmcgdGhlIG51bWJlciBhbmQgdGhlIGRpYW1ldGVyIG9mIHRoZSBob2xlcyBvZiB0aGUgcGVyZm9yYXRlZCBwbGF0ZSBpbiB0aGlzIHN0dWR5LiBUaGVuLCBhbiBleHBlcmltZW50YWwgc3lzdGVtIHdoZXJlIG94eWdlbiBhbmQgd2F0ZXIgd2VyZSBzZXBhcmF0ZWx5IGludHJvZHVjZWQgaW5zaWRlIHRoZSBwcm9wb3NlZCBuYW5vYnViYmxlIGdlbmVyYXRvciB3YXMgZGVzaWduZWQuIFRoZSBkaXNzb2x2ZWQgb3h5Z2VuIGNvbmNlbnRyYXRpb24gd2FzIG1lYXN1cmVkIHVuZGVyIGRpZmZlcmVudCBjb25kaXRpb25zLiBGaW5hbGx5LCB0aGUgcHJvZHVjZWQgSENPVyB3YXMgdXNlZCB0byBjdWx0aXZhdGUgYSBtaW5pLXN1bmZsb3dlci4gVGhyb3VnaCBhIHNlcmllcyBvZiBleHBlcmltZW50cywgaXQgd2FzIGZvdW5kIHRoYXQgd2l0aCB0aGUgaW1wcm92ZWQgcGVyZm9yYXRlZCBwbGF0ZSwgdGhlIGRpc3NvbHZlZCBveHlnZW4gY29uY2VudHJhdGlvbiB3YXMgaW5jcmVhc2VkIGFuZCB0aGUgbmFub2J1YmJsZSBnZW5lcmF0b3IgcmVhY2hlZCB0aGUgc2F0dXJhdGlvbiBzdGF0ZSBxdWlja2x5LCB3aGlsZSB0aGUgbWluaS1zdW5mbG93ZXIgY3VsdGl2YXRlZCB3aXRoIHRoZSBIQ09XIGFwcGVhcmVkIHRvIGdyb3cgbGFyZ2VyIHRoYW4gdGhhdCB3aXRoIHRhcCB3YXRlci4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMzM5MC9lbmdwcm9jMjAyMzA1NTAyMyIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cubWRwaS5jb20vMjY3My00NTkxLzU1LzEvMjMvcGRmP3ZlcnNpb249MTcwMTI0MzExOCIsImlzc3VlZERhdGUiOiIyMDIzLTExLTI5IiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInZvbHVtZSI6MzN9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjIzLCJsYXN0UGFnZSI6MjN9fSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjIsImNpdGVkQnlDb3VudCI6MX19LCJxdW90ZSI6IkFsdG1ldHJpY1tfc2hhcmVfIFNoYXJlXShodHRwczovL3d3dy5tZHBpLmNvbS8yNjczLTQ1OTEvNTUvMS8yMykgW19hbm5vdW5jZW1lbnRfIEhlbHBdKGh0dHBzOi8vd3d3Lm1kcGkuY29tLzI2NzMtNDU5MS81NS8xLzIzKSBbX2Zvcm1hdFxcX3F1b3RlXyBDaXRlXShqYXZhc2NyaXB0OnZvaWQoMCk7KSBbX3F1ZXN0aW9uXFxfYW5zd2VyXyBEaXNjdXNzIGluIFNjaVByb2ZpbGVzXShodHRwczovL3NjaXByb2ZpbGVzLmNvbS9kaXNjdXNzaW9uLWdyb3Vwcy9wdWJsaWMvMTAuMzM5MC9lbmdwcm9jMjAyMzA1NTAyMz91dG1fc291cmNlPW1wZGkuY29tJnV0bV9tZWRpdW09cHVibGljYXRpb24mdXRtX2NhbXBhaWduPWRpc2N1c3NfaW5fc2NpcHJvZmlsZXMpIFdhdGVyIHdpdGggYSBoaWdoIGNvbmNlbnRyYXRpb24gb2Ygb3h5Z2VuIGlzIG5lZWRlZCBmb3IgdGhlIGFxdWFjdWx0dXJlIGluZHVzdHJ5IGluIEphcGFuLiBJbiB0aGUgY3VycmVudCBzdHVkeSwgdGhlIHByZXNzdXJpemVkIGRpc3NvbHV0aW9uIG1ldGhvZCB3YXMgZW1wbG95ZWQgdG8gZ2VuZXJhdGUgaGlnaC1jb25jZW50cmF0aW9uIG94eWdlbmF0ZWQgd2F0ZXIgKEhDT1cpIGJ5IHByb2R1Y2luZyBveHlnZW4gbmFub2J1YmJsZXMgaW4gdGhlIHdhdGVyLiBJbiBvcmRlciB0byBpbnZlc3RpZ2F0ZSBmYWN0b3JzIHN1Y2ggYXMgdGVtcGVyYXR1cmUsIGdlb21ldHJpYyBjb25kaXRpb25zLCBhbmQgdGhlaXIgaW5mbHVlbmNlIG9uIHRoZSBveHlnZW4gY29uY2VudHJhdGlvbiwgYSBzcGVjaWFsIG5hbm9idWJibGUgZ2VuZXJhdG9yIHdhcyBpbXByb3ZlZCBieSBjaGFuZ2luZyB0aGUgbnVtYmVyIGFuZCB0aGUgZGlhbWV0ZXIgb2YgdGhlIGhvbGVzIG9mIHRoZSBwZXJmb3JhdGVkIHBsYXRlIGluIHRoaXMgc3R1ZHkuIn1d. Economic analyses indicate positive net present value (NPV: IDR 64,824,374) and short payback periods (1.7 years) for P. vannamei farms, underscoring feasibility (Susanti et al., 2021)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkxpbHkiLCJmYW1pbHkiOiJTdXNhbnRpIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlN1eXVkIFdhcm5vIiwiZmFtaWx5IjoiVXRvbW8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTm92ZXJpdGEgRGlhbiIsImZhbWlseSI6IlRha2FyaW5hIn19XSwiaWRzIjp7Imlzc24iOlsiMjExNy00NDU4IiwiMjI3My0xNzA5Il0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDUxL2Jpb2NvbmYvMjAyMTMzMDUwMDUiLCJtYWciOiIzMTkzNTYzNzAzIiwib3BlbmFsZXgiOiJXMzE5MzU2MzcwMyJ9LCJ0aXRsZSI6IlN1c3RhaW5hYmlsaXR5IGFuZCBmZWFzaWJpbGl0eSBhc3Nlc3NtZW50cyBvZiBuYW5vYnViYmxlIGFlcmF0aW9uIHRlY2hub2xvZ3kgaW4gZWNvbm9taWMtc29jaW8gZW52aXJvbm1lbnQgb2YgUGVuYWV1cyB2YW5uYW1laSBzaHJpbXAgZmFybWluZyIsImFic3RyYWN0IjoiUGVuYWV1cyB2YW5uYW1laSBzaHJpbXAgZmFybWluZyBpbiBJbmRvbmVzaWEgaXMgZmFjZWQgd2l0aCBzZXZlcmFsIGNoYWxsZW5nZXMgaW4gdGhlIGVudmlyb25tZW50YWwsIGVjb25vbWljLCBhbmQgc29jaWFsIGFzcGVjdHMuIFRoZXJlZm9yZSwgdGhpcyBzdHVkeSBhaW1zIHRvIGFzc2VzcyB0aGUgYmVuZWZpdHMgb2Ygbm92ZWwgbmFub2J1YmJsZSBhZXJhdGlvbiBzeXN0ZW1zIGZvciB0aGUgZWNvc3lzdGVtLCBidXNpbmVzc2VzLCBhbmQgY29tbXVuaXRpZXMuIFRoaXMgaXMgYW4gZXhwZXJpbWVudGFsIHN0dWR5IGNvbmR1Y3RlZCBieSBjb21wYXJpbmcgUC4gdmFubmFtZWkgcG9zdCBsYXJ2YSAxMCByZWFyZWQgYXQgYSBkZW5zaXR5IG9mIDIwMDAgaW5kcy4vTCBpbiBhIHBvbmQgdHJlYXRlZCB3aXRoIG5hbm9idWJibGUgYW5kIHdpdGhvdXQgdHJlYXRtZW50cyAoY29udHJvbCkuIEZ1cnRoZXJtb3JlLCB0aGUgTmV0IFByZXNlbnQgVmFsdWUgKE5QVikgYW5kIHBheWJhY2sgcGVyaW9kIHdlcmUgY2FsY3VsYXRlZCB0byBhc3Nlc3MgdGhlIGVjb25vbWljIGZlYXNpYmlsaXR5IG9mIG5hbm9idWJibGUsIHdoaWxlIGNvbW11bml0eSBpbnRlcnZpZXdzIHdlcmUgdXNlZCBmb3IgdGhlIHNvY2lhbCBzZWN0b3JzLiBCYXNlZCBvbiB0aGUgcmVzdWx0cywgZW52aXJvbm1lbnRhbCB3YXN0ZXMgb2Ygc2hyaW1wIGZhcm1pbmcgaW4gZm9ybSBvZiB0b3RhbCBhbW1vbmlhIG5pdHJvZ2VuIChUQU4pIHdlcmUgcmVkdWNlZCBieSA5JSBmcm9tIDIuNTggbWcvbCAoOTUlQ0k6IDAsOTEsIDQsMjUpIGluIGNvbnRyb2wgY29tcGFyZWQgdG8gMi4zNSBtZy9sICg5NSVDSTogMCw4NiwgMyw4NCkgaW4gdHJlYXRtZW50LiBGdXJ0aGVybW9yZSwgdGhlIHJldmVudWVzIGZyb20gcG9zdCBsYXJ2YSBzYWxlcyBmb3IgZml2ZSB5ZWFycyBhbmQgbmFub2J1YmJsZSBpbnZlc3RtZW50IGNvc3RzIHNob3dlZCB0aGF0IHRoZSBlc3RpbWF0ZWQgTlBWIHdhcyBJRFIgNjQsODI0LDM3NCB3aXRoIGEgcGF5YmFjayBwZXJpb2Qgb2YgMS43IHllYXJzLiBUaGUgaW50ZXJ2aWV3cyBvbiB0cmFkaXRpb25hbCBzaHJpbXAgZmFybWluZyBzaG93ZWQgdGhhdCA2MS4xJSB0byA3Mi4yJSBvZiBjb21tdW5pdHkgbWVtYmVycyBhZ3JlZWQgb24gdGhlIHVzZSBvZiBuYW5vYnViYmxlIGFlcmF0aW9uIHRvIHN1cHBvcnQgbGl2ZWxpaG9vZHMuIFRoZXJlZm9yZSwgdGhlIHVzZSBvZiBuYW5vYnViYmxlcyBpcyBmZWFzaWJsZSB0byBzdXBwb3J0IHN1c3RhaW5hYmxlIFAuIHZhbm5hbWVpIGZhcm1pbmcuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjEwNTEvYmlvY29uZi8yMDIxMzMwNTAwNSIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cuYmlvLWNvbmZlcmVuY2VzLm9yZy9hcnRpY2xlcy9iaW9jb25mL3BkZi8yMDIxLzA1L2Jpb2NvbmZfaWNhdmVzczIwMjFfMDUwMDUucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjEtMDEtMDEiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA5IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJCSU8gV2ViIG9mIENvbmZlcmVuY2VzIiwidm9sdW1lIjozMywiaW1wYWN0RmFjdG9yIjoiMC42MiJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjUwMDUsImxhc3RQYWdlIjo1MDA1fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkVEUCBTY2llbmNlcyJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MTEsImNpdGVkQnlDb3VudCI6NH19LCJxdW90ZSI6IkFic3RyYWN0LiBQZW5hZXVzIHZhbm5hbWVpIHNocmltcCBmYXJtaW5nIGluIEluZG9uZXNpYSBpcyBmYWNlZCB3aXRoIHNldmVyYWwgY2hhbGxlbmdlcyBpbiB0aGUgZW52aXJvbm1lbnRhbCwgZWNvbm9taWMsIGFuZCBzb2NpYWwgYXNwZWN0cy4gIFRoZXJlZm9yZSwgdGhpcyBzdHVkeSBhaW1zIHRvIGFzc2VzcyB0aGUgYmVuZWZpdHMgb2Ygbm92ZWwgbmFub2J1YmJsZSAgYWVyYXRpb24gc3lzdGVtcyBmb3IgdGhlIGVjb3N5c3RlbSwgYnVzaW5lc3NlcywgYW5kIGNvbW11bml0aWVzLiBUaGlzIGlzIGFuICBleHBlcmltZW50YWwgc3R1ZHkgY29uZHVjdGVkIGJ5IGNvbXBhcmluZyBQLiB2YW5uYW1laSBwb3N0IGxhcnZhIDEwICByZWFyZWQgYXQgYSBkZW5zaXR5IG9mIDIwMDAgaW5kcy4vTCBpbiBhIHBvbmQgdHJlYXRlZCB3aXRoIG5hbm9idWJibGUgYW5kICB3aXRob3V0IHRyZWF0bWVudHMgKGNvbnRyb2wpLiBGdXJ0aGVybW9yZSwgdGhlIE5ldCBQcmVzZW50IFZhbHVlIChOUFYpIGFuZCAgcGF5YmFjayBwZXJpb2Qgd2VyZSBjYWxjdWxhdGVkIHRvIGFzc2VzcyB0aGUgZWNvbm9taWMgZmVhc2liaWxpdHkgb2YgIG5hbm9idWJibGUsIHdoaWxlIGNvbW11bml0eSBpbnRlcnZpZXdzIHdlcmUgdXNlZCBmb3IgdGhlIHNvY2lhbCBzZWN0b3JzLiAgQmFzZWQgb24gdGhlIHJlc3VsdHMsIGVudmlyb25tZW50YWwgd2FzdGVzIG9mIHNocmltcCBmYXJtaW5nIGluIGZvcm0gb2YgIHRvdGFsIGFtbW9uaWEgbml0cm9nZW4gKFRBTikgd2VyZSByZWR1Y2VkIGJ5IDklIGZyb20gMi41OCBtZy9sICg5NSVDSTogIDAsOTEsIDQsMjUpIGluIGNvbnRyb2wgY29tcGFyZWQgdG8gMi4zNSBtZy9sICg5NSVDSTogMCw4NiwgMyw4NCkgaW4gIHRyZWF0bWVudC4gRnVydGhlcm1vcmUsIHRoZSByZXZlbnVlcyBmcm9tIHBvc3QgbGFydmEgc2FsZXMgZm9yIGZpdmUgeWVhcnMgYW5kICBuYW5vYnViYmxlIGludmVzdG1lbnQgY29zdHMgc2hvd2VkIHRoYXQgdGhlIGVzdGltYXRlZCBOUFYgd2FzIElEUiAgNjQsODI0LDM3NCB3aXRoIGEgcGF5YmFjayBwZXJpb2Qgb2YgMS43IHllYXJzLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjQ1LjQ4MzI4NzgxMTI3OTMsImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2LjgwMDU2NzYyNjk1MzEyNSwiaGVpZ2h0IjoxLjc0OTIzODYxMDI2NzYzOTJ9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo0Ny4xNjcxNzE0NzgyNzE0ODQsImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2Ljc3NTAzOTY3Mjg1MTU2LCJoZWlnaHQiOjEuNDA2Nzg1OTY0OTY1ODIwM319LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjQ4LjYyMzgwOTgxNDQ1MzEyNSwibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuNzcxMzAxMjY5NTMxMjUsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NTAuMDgwNDQ4MTUwNjM0NzY2LCJsZWZ0IjoyMS44Mjc3NzAyMzMxNTQyOTcsIndpZHRoIjo1Ni44MjAyOTcyNDEyMTA5NCwiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo1MS41MzcwOTAzMDE1MTM2NywibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuODI2NTMwNDU2NTQyOTcsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NTMuMDEwNjY1ODkzNTU0NjksImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2Ljc2NTcyNDE4MjEyODkwNiwiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo1NC40NjczMDQyMjk3MzYzMywibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuNzkzNzEyNjE1OTY2OCwiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo1NS45MjM5NDYzODA2MTUyMzQsImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2Ljc5NTYxNjE0OTkwMjM0NCwiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo1Ny4zODA1ODg1MzE0OTQxNCwibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuNzc2OTI3OTQ3OTk4MDUsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NTguODU0MTY0MTIzNTM1MTU2LCJsZWZ0IjoyMS44Mjc3NzAyMzMxNTQyOTcsIndpZHRoIjo1Ni43ODA2NjYzNTEzMTgzNiwiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2MC4zMTA3OTg2NDUwMTk1MywibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuODMzMzc0MDIzNDM3NSwiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2MS43Njc0NDA3OTU4OTg0NCwibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuNzQ3MDA5Mjc3MzQzNzUsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NjMuMjI4MzEzNDQ2MDQ0OTIsImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2Ljc4ODEyNzg5OTE2OTkyLCJoZWlnaHQiOjEuNDA2Nzg1OTY0OTY1ODIwM319LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjY0LjcwMTg4OTAzODA4NTk0LCJsZWZ0IjoyMS44Mjc3NzAyMzMxNTQyOTcsIndpZHRoIjo1Ni43ODYyNjI1MTIyMDcwMywiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2Ni4xNTg1MzExODg5NjQ4NCwibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuODE1MzE5MDYxMjc5MywiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI1MDA1In19XQ%3D%3D. However, optimal performance requires neutral-alkaline pH to maximize zeta potential and OH⁻ adsorption, aligning with typical pond conditions (Wu et al., 2021)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjMwLCJzdWJ0eXBlIjp7fSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSmlhamlhIiwiZmFtaWx5IjoiV3UifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiS2VqaWEiLCJmYW1pbHkiOiJaaGFuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJDaGVuZyIsImZhbWlseSI6IkNlbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJYaWFvZ2FuZyIsImZhbWlseSI6Ild1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlJ1eWluIiwiZmFtaWx5IjoiTWFvIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Illpbmd5aW5nIiwiZmFtaWx5IjoiWmhlbmcifX1dLCJpZHMiOnsiaXNzbiI6WyIyMTkxLTA4NTUiXSwiaXNibiI6W10sImRvaSI6IjEwLjExODYvczEzNTY4LTAyMS0wMTI1NC0wIiwibWFnIjoiMzE3NTY4MzU2MSIsInBtaWQiOiIzNDE4NDEzNyIsInBtY2lkIjoiODIzOTEwOSIsIm9wZW5hbGV4IjoiVzMxNzU2ODM1NjEifSwidGl0bGUiOiJSb2xlIG9mIGJ1bGsgbmFub2J1YmJsZXMgaW4gcmVtb3Zpbmcgb3JnYW5pYyBwb2xsdXRhbnRzIGluIHdhc3Rld2F0ZXIgdHJlYXRtZW50IiwiYWJzdHJhY3QiOiJBYnN0cmFjdCBUaGUgb2NjdXJyZW5jZSBvZiBhIHZhcmlldHkgb2Ygb3JnYW5pYyBwb2xsdXRhbnRzIGhhcyBjb21wbGljYXRlZCB3YXN0ZXdhdGVyIHRyZWF0bWVudDsgdGh1cywgdGhlIHNlYXJjaCBmb3Igc3VzdGFpbmFibGUgYW5kIGVmZmVjdGl2ZSB0cmVhdG1lbnQgdGVjaG5vbG9neSBoYXMgZHJhd24gc2lnbmlmaWNhbnQgYXR0ZW50aW9uLiBJbiByZWNlbnQgeWVhcnMsIGJ1bGsgbmFub2J1YmJsZXMsIHdoaWNoIGhhdmUgZXh0cmFvcmRpbmFyeSBwcm9wZXJ0aWVzIGRpZmZlcmluZyBmcm9tIHRob3NlIG9mIG1pY3JvYnViYmxlcywgaW5jbHVkaW5nIGhpZ2ggc3RhYmlsaXR5IGFuZCBsb25nIHJlc2lkZW5jZSB0aW1lcyBpbiB3YXRlciwgbGFyZ2Ugc3BlY2lmaWMgc3VyZmFjZSBhcmVhcywgaGlnaCBnYXMgdHJhbnNmZXIgZWZmaWNpZW5jeSBhbmQgaW50ZXJmYWNlIHBvdGVudGlhbCwgYW5kIHRoZSBjYXBhYmlsaXR5IHRvIGdlbmVyYXRlIGZyZWUgcmFkaWNhbHMsIGhhdmUgc2hvd24gYXR0cmFjdGl2ZSB0ZWNobm9sb2dpY2FsIGFkdmFudGFnZXMgYW5kIHByb21pc2luZyBhcHBsaWNhdGlvbiBwcm9zcGVjdHMgZm9yIHdhc3Rld2F0ZXIgdHJlYXRtZW50LiBJbiB0aGlzIHJldmlldywgdGhlIGJhc2ljIGNoYXJhY3RlcmlzdGljcyBvZiBidWxrIG5hbm9idWJibGVzIGFyZSBzdW1tYXJpemVkIGluIGRldGFpbCwgYW5kIHJlY2VudCBmaW5kaW5ncyByZWxhdGVkIHRvIHRoZWlyIGltcGxlbWVudGF0aW9uIHBhdGh3YXlzIGFuZCBtZWNoYW5pc21zIGluIG9yZ2FuaWMgd2FzdGV3YXRlciB0cmVhdG1lbnQgYXJlIHN5c3RlbWF0aWNhbGx5IGRpc2N1c3NlZCwgd2hpY2ggaW5jbHVkZXMgaW1wcm92aW5nIHRoZSBhaXIgZmxvdGF0aW9uIHByb2Nlc3MsIGluY3JlYXNpbmcgd2F0ZXIgYWVyYXRpb24gdG8gcHJvbW90ZSBhZXJvYmljIGJpb2xvZ2ljYWwgdGVjaG5vbG9naWVzIGluY2x1ZGluZyBiaW9sb2dpY2FsIGFjdGl2YXRlZCBjYXJib24sIGFjdGl2YXRlZCBzbHVkZ2UsIGFuZCBtZW1icmFuZSBiaW9yZWFjdG9ycywgYW5kIGdlbmVyYXRpbmcgYWN0aXZlIGZyZWUgcmFkaWNhbHMgdGhhdCBveGlkaXNlIG9yZ2FuaWMgY29tcG91bmRzLiBGaW5hbGx5LCB0aGUgY3VycmVudCB0ZWNobm9sb2dpY2FsIGRpZmZpY3VsdGllcyBvZiBidWxrIG5hbm9idWJibGVzIGFyZSBhbmFseXNlZCwgYW5kIGZ1dHVyZSBmb2N1cyBhcmVhcyBmb3IgcmVzZWFyY2ggb24gYnVsayBuYW5vYnViYmxlIHRlY2hub2xvZ3kgYXJlIGFsc28gcHJvcG9zZWQuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjExODYvczEzNTY4LTAyMS0wMTI1NC0wIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2FtYi1leHByZXNzLnNwcmluZ2Vyb3Blbi5jb20vY291bnRlci9wZGYvMTAuMTE4Ni9zMTM1NjgtMDIxLTAxMjU0LTAiLCJpc3N1ZWREYXRlIjoiMjAyMS0wNi0yOCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkFNQiBFeHByZXNzIiwiaXNzdWUiOjEsInZvbHVtZSI6MTEsImltcGFjdEZhY3RvciI6IjQuMjYifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiU3ByaW5nZXIgTmF0dXJlIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo3NiwiY2l0ZWRCeUNvdW50Ijo3NX19LCJxdW90ZSI6IlRoZSBtb2RlbCBzdWNjZXNzZnVsbHkgZWx1Y2lkYXRlcyByZXB1bHNpdmUgZm9yY2VzICBiZXR3ZWVuIE9I4oiSIG9uIHRoZSBzdXJmYWNlIGFyZSBzdWZmaWNpZW50IHRvIHVsdGktbWF0ZWx5IGJhbGFuY2UgdGhlIHN1cmZhY2UgdGVuc2lvbiBmb3JjZXMgYXQgbmFub2J1YmJsZSAgc2l6ZXMsIHRodXMsIG1ha2luZyBuYW5vYnViYmxlcyBleGhpYml0IGdyZWF0IHN0YWJpbC1pdHkuIEFzIHNob3duIGluIHByZXZpb3VzIGxpdGVyYXR1cmVzLCB6ZXRhIHBvdGVudGlhbCB2YWwtdWVzIGNoYW5nZSB3aXRoIHRoZSBwSCBvZiB0aGUgbWVkaXVtIHZhcnksIGV4aGliaXRpbmcgIGEgdGVuZGVuY3kgdGhhdCB0aGUgbmVnYXRpdml0eSBvZiB6ZXRhIHBvdGVudGlhbCBpbmNyZWFzZXMgIHdpdGggc29sdXRpb24gcEggaW5jcmVhc2Ugd2hpY2ggcmVzdWx0cyBpbiBncmVhdGVyIGJ1Yi1ibGUgc3VyZmFjZSBjaGFyZ2UgYW5kIG1ha2UgYnViYmxlcyBtb3JlIHN0YWJsZSBkdWUgdG8gIGVsZWN0cm9zdGF0aWMgcmVwdWxzaW9uIChNZWVnb2RhIGV0IGFsLiAyMDE4KS4gTmFub2J1Yi1ibGVzIGdlbmVyYWxseSBzaG93IGhpZ2ggbmVnYXRpdmUgemV0YSBwb3RlbnRpYWwgb3dpbmcgdG8gIHRoZSBleGNlc3Mgb2YgaHlkcm94aWRlIGlvbnMgKE9I4oiSKSByZWxhdGl2ZSB0byBoeWRyb2dlbiAgaW9ucyAoSCspIGF0IHRoZSBnYXPigJN3YXRlciBpbnRlcmZhY2Ugb2YgYnViYmxlcyB1bmRlciAgd2lkZSByYW5nZSBvZiBzb2x1dGlvbiBwSCwgYnV0IG5hbm9idWJibGVzIGZvcm1lZCBpbiAgYWNpZGljIGVudmlyb25tZW50cyBhbHdheXMgc2hvdyBsb3dlciBzdGFiaWxpdHkgY29tLXBhcmVkIHdpdGggaW4gbmV1dHJhbCBvciBhbGthbGluZSBjb25kaXRpb25zIGJlY2F1c2UgYSAgbGFjayBvZiBPSOKIkiBhcmUgcmVxdWlyZWQgdG8gc3RhYmlsaXplIHRoZSBlbGVjdHJpYyBkb3VibGUgIGxheWVyIChOaXJtYWxrYXIgZXQgYWwuIDIwMTgpLCB0aGUgemV0YSBwb3RlbnRpYWwgb2YgbmFuby1idWJibGVzIGdlbmVyYXRlZCBpbiBzdHJvbmcgYWNpZGljIG1lZGl1bSBiZWxvdyBwSCAzICB0ZW5kcyB0byBiZSBwb3NpdGl2ZSBhbmQgbG93IHdoaWNoIGNhdXNlcyBoaWdoZXIgcG9zc2ktYmlsaXR5IGZvciBidWJibGUgY29hbGVzY2VuY2UgKFpoYW5nIGV0IGFsLiAyMDIwKS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjoxMy45NjgzMDU1ODc3Njg1NTUsImxlZnQiOjUyLjUzNDM0NzUzNDE3OTY5LCJ3aWR0aCI6MzguMzI2OTE1NzQwOTY2OCwiaGVpZ2h0IjoxLjgzNzY1NjYxNzE2NDYxMTh9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjoxNS40NzczMzQwMjI1MjE5NzMsImxlZnQiOjUxLjE5MDk3MTM3NDUxMTcyLCJ3aWR0aCI6MzkuMjg1MTYzODc5Mzk0NTMsImhlaWdodCI6MS44NDY1NjE1NTEwOTQwNTUyfX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MTcuMDAzNTYyOTI3MjQ2MDk0LCJsZWZ0Ijo1MS4xOTA0MzczMTY4OTQ1Mywid2lkdGgiOjM5LjY2OTEwNTUyOTc4NTE1NiwiaGVpZ2h0IjoxLjgzNzY1NjYxNzE2NDYxMTh9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjoxOC41MjAyNzcwMjMzMTU0MywibGVmdCI6NTEuMTkwNDM3MzE2ODk0NTMsIndpZHRoIjozOS4yODU2OTQxMjIzMTQ0NSwiaGVpZ2h0IjoxLjgzODI2NjI1MzQ3MTM3NDV9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjoyMC4wMzgyMTE4MjI1MDk3NjYsImxlZnQiOjUxLjE5MDQzNzMxNjg5NDUzLCJ3aWR0aCI6MzkuMjg1Njk0MTIyMzE0NDUsImhlaWdodCI6MS44Mzc2NTY2MTcxNjQ2MTE4fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MjEuNTU1NTM2MjcwMTQxNiwibGVmdCI6NTEuMTkwNDM3MzE2ODk0NTMsIndpZHRoIjozOS42NzUyMTY2NzQ4MDQ2OSwiaGVpZ2h0IjoxLjgzNzY1NjYxNzE2NDYxMTh9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjoyMy4wNzIyNDg0NTg4NjIzMDUsImxlZnQiOjUxLjE5MDQzNzMxNjg5NDUzLCJ3aWR0aCI6MzkuNjcwMzI2MjMyOTEwMTU2LCJoZWlnaHQiOjEuODM3NjU2NjE3MTY0NjExOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjI0LjU4ODk2MjU1NDkzMTY0LCJsZWZ0Ijo1MS4xOTA0MzczMTY4OTQ1Mywid2lkdGgiOjM5LjI4NTY5NDEyMjMxNDQ1LCJoZWlnaHQiOjEuODM4ODc1ODg5Nzc4MTM3Mn19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjI2LjEwNzUwNzcwNTY4ODQ3NywibGVmdCI6NTEuMTkwNDM3MzE2ODk0NTMsIndpZHRoIjozOS42NzIyNTI2NTUwMjkzLCJoZWlnaHQiOjEuODM3NjU2NjE3MTY0NjExOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjI3LjYyNDIyMTgwMTc1NzgxMiwibGVmdCI6NTEuMTkwNDM3MzE2ODk0NTMsIndpZHRoIjozOS4yODU2OTQxMjIzMTQ0NSwiaGVpZ2h0IjoxLjgzODI2NjI1MzQ3MTM3NDV9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjoyOS4xNDIxNTY2MDA5NTIxNSwibGVmdCI6NTEuMTkwNDM3MzE2ODk0NTMsIndpZHRoIjozOS42NzUyMzU3NDgyOTEwMTYsImhlaWdodCI6MS44Mzc2NTY2MTcxNjQ2MTE4fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MzAuNjUwNTczNzMwNDY4NzUsImxlZnQiOjUxLjE5MDQzNzMxNjg5NDUzLCJ3aWR0aCI6MzkuNjcxNzI2MjI2ODA2NjQsImhlaWdodCI6MS44NDY1NTg4MDkyODAzOTU1fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MzIuMTY3OTAwMDg1NDQ5MjIsImxlZnQiOjUxLjE4OTg5OTQ0NDU4MDA4LCJ3aWR0aCI6MzkuNjY5OTQwOTQ4NDg2MzMsImhlaWdodCI6MS44NDY1NTg4MDkyODAzOTU1fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MzMuNjkzNTE1Nzc3NTg3ODksImxlZnQiOjUxLjE5MDg2MDc0ODI5MTAxNiwid2lkdGgiOjM5LjY3NTcyMDIxNDg0Mzc1LCJoZWlnaHQiOjEuODM3NjU0NTkwNjA2Njg5NX19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjM1LjIxMDIzNTU5NTcwMzEyNSwibGVmdCI6NTEuMTkwODYwNzQ4MjkxMDE2LCJ3aWR0aCI6MzkuMjg1MjcwNjkwOTE3OTcsImhlaWdodCI6MS44Mzg4NzQxMDE2Mzg3OTR9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjozNi43Mjg3Nzg4MzkxMTEzMywibGVmdCI6NTEuMTkwNDM3MzE2ODk0NTMsIndpZHRoIjozOS42Njg5NzIwMTUzODA4NiwiaGVpZ2h0IjoxLjgzNzY1NDU5MDYwNjY4OTV9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjozOC4yMzcxNzg4MDI0OTAyMzQsImxlZnQiOjUxLjE5MDQzNzMxNjg5NDUzLCJ3aWR0aCI6MzkuNjY4ODA3OTgzMzk4NDQsImhlaWdodCI6MS44NDY1ODAwMjg1MzM5MzU1fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MzkuNzYyODIxMTk3NTA5NzY2LCJsZWZ0Ijo1MS4xODk5NTI4NTAzNDE4LCJ3aWR0aCI6MzkuMjg2MTgyNDAzNTY0NDUsImhlaWdodCI6MS44MzgyNjA1MzE0MjU0NzZ9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo0MS4yODA3NTAyNzQ2NTgyLCJsZWZ0Ijo1MS4xOTA0MzczMTY4OTQ1Mywid2lkdGgiOjM5LjY3MjczMzMwNjg4NDc2NiwiaGVpZ2h0IjoxLjgzNzY1NDU5MDYwNjY4OTV9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo0Mi43OTc0NzAwOTI3NzM0NCwibGVmdCI6NTEuMTkwNDM3MzE2ODk0NTMsIndpZHRoIjozOS4yODU2OTQxMjIzMTQ0NSwiaGVpZ2h0IjoxLjgzODI2MDUzMTQyNTQ3Nn19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjQ0LjMxNTM5OTE2OTkyMTg3NSwibGVmdCI6NTEuMTkwNDM3MzE2ODk0NTMsIndpZHRoIjozOS4yODU2OTQxMjIzMTQ0NSwiaGVpZ2h0IjoxLjgzNzY1NDU5MDYwNjY4OTV9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIzIn19XQ%3D%3D.

In summary, nanobubbles improve aquaculture water quality by elevating DO, degrading nitrogenous wastes via ROS, and fostering stable physicochemical profiles, outperforming conventional methods in efficiency and sustainability. Their integration promises reduced energy use and environmental footprints, paving the way for resilient farming practices (Tekile et al., 2016)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFuZGluZXQgS2ViZWRlIiwiZmFtaWx5IjoiVGVraWxlIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikls4oCQSG8iLCJmYW1pbHkiOiJLaW0ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSmFpeWVvcCIsImZhbWlseSI6IkxlZSJ9fV0sImlkcyI6eyJpc3NuIjpbIjEyMjYtMTAyNSIsIjIwMDUtOTY4WCJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuNDQ5MS9lZXIuMjAxNi4wMjgiLCJtYWciOiIyNTIzNjY5NjU3Iiwib3BlbmFsZXgiOiJXMjUyMzY2OTY1NyJ9LCJ0aXRsZSI6IkV4dGVudCBhbmQgcGVyc2lzdGVuY2Ugb2YgZGlzc29sdmVkIG94eWdlbiBlbmhhbmNlbWVudCB1c2luZyBuYW5vYnViYmxlcyIsImFic3RyYWN0IjoiSW4gdGhpcyBzdHVkeSwgY2hhbmdlIGluIHdhdGVyLWRpc3NvbHZlZCBveHlnZW4gKERPKSB3YXMgYW5hbHl6ZWQgdW5kZXIgdmFyaW91cyBzeW50aGV0aWMgd2F0ZXIgcXVhbGl0aWVzIGFuZCBuYW5vYnViYmxlcyAoTkJzKSBhcHBsaWNhdGlvbiBjb25kaXRpb25zLCBzdWNoIGFzIGdhcyB0eXBlLCBpbml0aWFsIERPIGFzIHdlbGwgYXMgd2F0ZXIgZGlzc29sdmVkLCBzdXNwZW5kZWQgYW5kIG9yZ2FuaWMgbWF0dGVycyBjb250ZW50cy4gV2hlbiBveHlnZW4sIHJhdGhlciB0aGFuIGFpciwgd2FzIGludHJvZHVjZWQgaW50byBuaXRyb2dlbi1kZXNvcmJlZCB1bHRyYS1wdXJlIHdhdGVyLCB0aGUgc3RhZ25hdGlvbiB0aW1lIHdhcyBzaWduaWZpY2FudGx5IGluY3JlYXNlZC4gSXQgdG9vayB0ZW4gZGF5cyBmb3IgRE8gY29uY2VudHJhdGlvbiB0byBkcm9wIGJhY2sgdG8gc2F0dXJhdGlvbi4gVGhlIGhpZ2hlciB0aGUgaW5pdGlhbCBETyBjb25jZW50cmF0aW9uLCB0aGUgbG9uZ2VyIHBhcnRpY2xlcyB3ZXJlIG9ic2VydmVkIGFib3ZlIHNhdHVyYXRpb24gZHVlIHRvIHBhcnRpY2xlIHN0YWJpbGl0eSBpbXByb3ZlbWVudC4gVGhlIG94eWdlbiBtYXNzIHRyYW5zZmVyIHJhdGUgb2YgMC4wNDgyIG1nL0wvbWluIHdhcyBmb3VuZCB0byByZWFjaCBhIG1heGltdW0gYXQgYW4gZWxlY3Ryb2x5dGljIGNvbmNlbnRyYXRpb24gb2YgMC43NSBnL0wsIGJleW9uZCB3aGljaCB0aGUgdHJhbnNmZXIgcmF0ZSBkZWNyZWFzZWQgZHVlIHRvIGFkc29ycHRpb24gb2YgbmVnYXRpdmUgaW9ucyBvZiB0aGUgZWxlY3Ryb2x5dGUgYXQgdGhlIGludGVyZmFjZS4gSGlnaCBsZXZlbHMgb2YgdHVyYmlkaXR5IGNhdXNlZCBieSBzdXNwZW5kZWQgc29saWRzIGhhdmUgYmVjb21lIGEgYmFycmllciB0byBkaXNzb2x1dGlvbiBvZiBOQnMgb3h5Z2VuIGludG8gdGhlIHdhdGVyIHNvbHV0aW9uLCBhbmQgdGh1cyBhZmZlY3RlZCB0aGUgdHJhbnNmZXIgcGVyZm9ybWFuY2UuIE9uIHRoZSBvdGhlciBoYW5kLCBieSBhcHBseWluZyBOQnMgZm9yIGp1c3QgYW4gaG91ciwgdXAgdG8gNy4yJSBkZWdyYWRhdGlvbiBvZiBnbHVjb3NlIGFzIHJlcHJlc2VudGF0aXZlIG9yZ2FuaWMgbWF0dGVyIHdhcyBhY2hpZXZlZC4gVGh1cywgTkJzIHRlY2hub2xvZ3kgd291bGQgbWFpbnRhaW4gYSBoaWdoIERPIGV4dGVudCBmb3IgYW4gZXh0ZW5kZWQgZHVyYXRpb24sIGFuZCB0aHVzIGNhbiBpbXByb3ZlIHdhdGVyIHF1YWxpdHkgcHJvdmlkZWQgdGhhdCB3YXRlciBjaGVtaXN0cnkgaXMgY2xvc2VseSBtb25pdG9yZWQgZHVyaW5nIGl0cyBhcHBsaWNhdGlvbi4gS2V5d29yZHM6IEFwcGxpY2F0aW9uIGNvbmRpdGlvbnMsIERpc3NvbHZlZCBveHlnZW4sIE5hbm9idWJibGVzLCBXYXRlciBxdWFsaXR5IiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjQ0OTEvZWVyLjIwMTYuMDI4IiwicmVzb3VyY2VVcmwiOiJodHRwOi8vZWVlci5vcmcvdXBsb2FkL2Vlci0yMS00LTQyNy5wZGYiLCJpc3N1ZWREYXRlIjoiMjAxNi0wOS0xMiIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkVudmlyb25tZW50YWwgRW5naW5lZXJpbmcgUmVzZWFyY2giLCJpc3N1ZSI6NCwidm9sdW1lIjoyMSwiaW1wYWN0RmFjdG9yIjoiNC4wNSJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjQyNywibGFzdFBhZ2UiOjQzNX19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJLb3JlYW4gU29jaWV0eSBvZiBFbnZpcm9ubWVudGFsIEVuZ2luZWVyaW5nIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjoyNCwiY2l0ZWRCeUNvdW50IjozM319LCJxdW90ZSI6IlRoaXMgdmVyaWZpZXMgdGhlIHBvc3NpYmlsaXR5IG9mIHRoZSBleGlzdGVuY2UgIG9mIE5Ccywgc2luY2UgYXQgYSBoaWdoZXIgRE8gY29uY2VudHJhdGlvbiwgdGhlIG94eWdlbiBkaWZmdXNpb24gIHdvdWxkIHRha2UgbW9yZSB0aW1lLiBUaGUgaGlnaCBnYXMgY29uY2VudHJhdGlvbiBpbiB0aGUgbGlxdWlkICBleHBsYWlucyB0aGUgZXh0ZW5zaW9uIG9mIE5CIHN0YWJpbGl0eS4gIFRoZSBtYWduaXR1ZGVzIG9mIHpldGEgcG90ZW50aWFsIG9mIG94eWdlbiBuYW5vLWJ1YmJsZWQgd2F0ZXIgIHdlcmUgcG9zc2libHkgaGlnaGVyIHRoYW4gdGhvc2Ugb2YgYWlyIG5hbm8tYnViYmxlZCB3YXRlci4gQXQgIGEgaGlnaCBtYWduaXR1ZGUgemV0YSBwb3RlbnRpYWwgdmFsdWUsIHRoZSBlbGVjdHJpY2FsbHkgY2hhcmdlZCAgcGFydGljbGVzIHRlbmQgdG8gcmVwZWwgZWFjaCBvdGhlciwgYXZvaWRpbmcgYWdncmVnYXRpb24uIEZvciBidWJibGUgIGRpc3BlcnNpb24sIHRoZSBoaWdoIHpldGEgcG90ZW50aWFsIGNvdWxkIGNyZWF0ZSByZXB1bHNpb24gZm9yY2VzICB0aGF0IHdvdWxkIGF2b2lkIHRoZSBjb2FsZXNjZW5jZSBvZiBidWJibGVzLCB0aHVzIGNvbnRyaWJ1dGluZyAgdG8gdGhlIHN0YWJpbGl6YXRpb24gb2YgdGhlIGJ1YmJsZXMuIFRoZXJlZm9yZSwgdGhlIG94eWdlbiBOQnMgIGNvdWxkIGJlIHN0YWJpbGl6ZWQgaW4gd2F0ZXIgYnkgdGhlIGVsZWN0cmljYWxseSBjaGFyZ2VkIHN1cmZhY2VzLCAgd2hpbGUgdGhlIG5hbm8tYnViYmxlZCBhaXIgd291bGQgYmUgYmVsb3cgdGhlIGxpbWl0IG9mIHRoZSBzdGFiaWxpemEtdGlvbiBieSByZXB1bHNpb24gZm9yY2VzLiBGdXJ0aGVybW9yZSwgdGhlIGJpZyBkaWZmZXJlbmNlIGJldHdlZW4gdGhlIHBlcmlvZHMgaW4gd2hpY2ggIGJ1YmJsZXMgY291bGQgYmUgb2JzZXJ2ZWQgYXMgc3RhYmxlIGluIHRoZSBveHlnZW4gbmFuby1idWJibGVkICB3YXRlciAoMTAtMTUgZCkgYW5kIGluIHRoYXQgb2YgYWlyIG5hbm8tYnViYmxlZCAoMy01IGQpIGlzIGxpa2VseSBGaWcuIDYuIEdyYXBocyBzaG93aW5nIHN0YWduYXRpb24gdGltZSBmb3IgZGlmZmVyZW50IGluaXRpYWwgZGlzc29sdmVkIG94eWdlbiAoRE8pLiByZWxhdGVkIHRvIHRoZSBsb3dlciBkaXNzb2x1dGlvbiBvZiB0aGUgYWlyIGluIHdhdGVyIGluIHJlbGF0aW9uIHRvIHRoZSBkaXNzb2x1dGlvbiBvZiBveHlnZW4uICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6NjYuMTc4ODAyNDkwMjM0MzgsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6NDAuNzE0ODg5NTI2MzY3MTksImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo2Ny41NzY3NzQ1OTcxNjc5NywibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42ODgyNzA1Njg4NDc2NTYsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo2OC45NjcyMzE3NTA0ODgyOCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NTcxODA3ODYxMzI4MSwiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjcwLjM1NzY4ODkwMzgwODYsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6MjUuMzQxMjU1MTg3OTg4MjgsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3MS43NTU2NTMzODEzNDc2NiwibGVmdCI6OS41NDk2NDE2MDkxOTE4OTUsIndpZHRoIjozOS4wMjQ3ODQwODgxMzQ3NjYsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3My4xNDYxMDI5MDUyNzM0NCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NTQwMjk4NDYxOTE0MDYsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3NC41MzY1NjAwNTg1OTM3NSwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NTM0NjUyNzA5OTYwOTQsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3NS45MzQ1MzIxNjU1MjczNCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NzMyMzY4NDY5MjM4MywiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjc3LjMyNDk4OTMxODg0NzY2LCJsZWZ0Ijo3Ljg2NTU0NTI3MjgyNzE0OCwid2lkdGgiOjQwLjY0OTM1MzAyNzM0Mzc1LCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6NzguNzE1NDQ2NDcyMTY3OTcsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6NDAuNjU0NjI0OTM4OTY0ODQ0LCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODAuMTEzNDEwOTQ5NzA3MDMsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6NDAuNjUxMDUwNTY3NjI2OTUsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo4MS41MDM4NjA0NzM2MzI4MSwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NTQzNDI2NTEzNjcxOSwiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjgyLjg5NDMxNzYyNjk1MzEyLCJsZWZ0Ijo3Ljg2NTU0NTI3MjgyNzE0OCwid2lkdGgiOjQwLjMyMTgyMzEyMDExNzE5LCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODQuMjkyMjg5NzMzODg2NzIsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6MTUuODcwMjk1NTI0NTk3MTY4LCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODUuNjgyNzQ2ODg3MjA3MDMsImxlZnQiOjkuNTQ5NjQxNjA5MTkxODk1LCJ3aWR0aCI6MzguOTg5NzA0MTMyMDgwMDgsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo4Ny4wNzMyMDQwNDA1MjczNCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC43MTg0OTQ0MTUyODMyLCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODguNDcxMTY4NTE4MDY2NCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42ODc0MTk4OTEzNTc0MiwiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjM0LjUxMzI3MTMzMTc4NzExLCJsZWZ0Ijo1MS45MzI3Nzc0MDQ3ODUxNTYsIndpZHRoIjo0MC42NDcwNTI3NjQ4OTI1OCwiaGVpZ2h0IjoyLjcxODY2NjU1MzQ5NzMxNDV9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjozOC43MjUxNTg2OTE0MDYyNSwibGVmdCI6NTEuOTMyNzc3NDA0Nzg1MTU2LCJ3aWR0aCI6NDAuNjQzMTM1MDcwODAwNzgsImhlaWdodCI6MS4yNTMyODg1MDc0NjE1NDc5fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6NDAuMTE1NjEyMDMwMDI5MywibGVmdCI6NTEuOTMyNzc3NDA0Nzg1MTU2LCJ3aWR0aCI6NDAuNjY0NTg1MTEzNTI1MzksImhlaWdodCI6MS4yNTMyODg1MDc0NjE1NDc5fX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNDMxIn19XQ%3D%3D. Furthermore, the antimicrobial potential of nanobubbles extends beyond water quality remediation to direct pathogen suppression, offering a chemical-free alternative to prophylactic antibiotics. The major aquaculture applications of nanobubbles and their underlying mechanisms are summarized in Table 2.

Table 2: Applications of nanobubbles in aquaculture

| Application | Mechanism | Benefits |
| --- | --- | --- |
| Water quality improvement | Enhanced gas transfer, oxidation of pollutants | Increased dissolved oxygen, reduced harmful substances |
| Disease control | Inactivation of pathogens through ROS generation | Reduced disease prevalence, improved fish health |
| Growth promotion | Increased oxygen availability, stimulation of beneficial microbes | Enhanced growth performance, feed conversion efficiency |

2.3.2. Disease Control

Nanobubbles exhibit potent antimicrobial activity, positioning them as a transformative tool for disease control in aquaculture systems plagued by bacterial, viral, and parasitic outbreaks. In high-density farming environments, pathogens such as Vibrio species, white spot syndrome virus in shrimp, and bacterial infections in finfish proliferate rapidly, leading to mass mortalities and economic losses. Unlike conventional antibiotics, which foster resistance and residue accumulation, nanobubbles leverage physical and chemical mechanisms-primarily reactive oxygen species generation and biophysical disruption-to inactivate microbes without chemical additives. Upon collapse, nanobubbles release hydroxyl radicals (•OH), superoxide (O₂⁻•), and hydrogen peroxide (H₂O₂) due to extreme localized pressures and temperatures, as described by the Young-Laplace equation (ΔP = 2σ/r), where smaller radii amplify internal stress  (Hutagalung et al., 2023; Wu et al., 2021)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjMwLCJzdWJ0eXBlIjp7fSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSmlhamlhIiwiZmFtaWx5IjoiV3UifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiS2VqaWEiLCJmYW1pbHkiOiJaaGFuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJDaGVuZyIsImZhbWlseSI6IkNlbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJYaWFvZ2FuZyIsImZhbWlseSI6Ild1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlJ1eWluIiwiZmFtaWx5IjoiTWFvIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Illpbmd5aW5nIiwiZmFtaWx5IjoiWmhlbmcifX1dLCJpZHMiOnsiaXNzbiI6WyIyMTkxLTA4NTUiXSwiaXNibiI6W10sImRvaSI6IjEwLjExODYvczEzNTY4LTAyMS0wMTI1NC0wIiwibWFnIjoiMzE3NTY4MzU2MSIsInBtaWQiOiIzNDE4NDEzNyIsInBtY2lkIjoiODIzOTEwOSIsIm9wZW5hbGV4IjoiVzMxNzU2ODM1NjEifSwidGl0bGUiOiJSb2xlIG9mIGJ1bGsgbmFub2J1YmJsZXMgaW4gcmVtb3Zpbmcgb3JnYW5pYyBwb2xsdXRhbnRzIGluIHdhc3Rld2F0ZXIgdHJlYXRtZW50IiwiYWJzdHJhY3QiOiJBYnN0cmFjdCBUaGUgb2NjdXJyZW5jZSBvZiBhIHZhcmlldHkgb2Ygb3JnYW5pYyBwb2xsdXRhbnRzIGhhcyBjb21wbGljYXRlZCB3YXN0ZXdhdGVyIHRyZWF0bWVudDsgdGh1cywgdGhlIHNlYXJjaCBmb3Igc3VzdGFpbmFibGUgYW5kIGVmZmVjdGl2ZSB0cmVhdG1lbnQgdGVjaG5vbG9neSBoYXMgZHJhd24gc2lnbmlmaWNhbnQgYXR0ZW50aW9uLiBJbiByZWNlbnQgeWVhcnMsIGJ1bGsgbmFub2J1YmJsZXMsIHdoaWNoIGhhdmUgZXh0cmFvcmRpbmFyeSBwcm9wZXJ0aWVzIGRpZmZlcmluZyBmcm9tIHRob3NlIG9mIG1pY3JvYnViYmxlcywgaW5jbHVkaW5nIGhpZ2ggc3RhYmlsaXR5IGFuZCBsb25nIHJlc2lkZW5jZSB0aW1lcyBpbiB3YXRlciwgbGFyZ2Ugc3BlY2lmaWMgc3VyZmFjZSBhcmVhcywgaGlnaCBnYXMgdHJhbnNmZXIgZWZmaWNpZW5jeSBhbmQgaW50ZXJmYWNlIHBvdGVudGlhbCwgYW5kIHRoZSBjYXBhYmlsaXR5IHRvIGdlbmVyYXRlIGZyZWUgcmFkaWNhbHMsIGhhdmUgc2hvd24gYXR0cmFjdGl2ZSB0ZWNobm9sb2dpY2FsIGFkdmFudGFnZXMgYW5kIHByb21pc2luZyBhcHBsaWNhdGlvbiBwcm9zcGVjdHMgZm9yIHdhc3Rld2F0ZXIgdHJlYXRtZW50LiBJbiB0aGlzIHJldmlldywgdGhlIGJhc2ljIGNoYXJhY3RlcmlzdGljcyBvZiBidWxrIG5hbm9idWJibGVzIGFyZSBzdW1tYXJpemVkIGluIGRldGFpbCwgYW5kIHJlY2VudCBmaW5kaW5ncyByZWxhdGVkIHRvIHRoZWlyIGltcGxlbWVudGF0aW9uIHBhdGh3YXlzIGFuZCBtZWNoYW5pc21zIGluIG9yZ2FuaWMgd2FzdGV3YXRlciB0cmVhdG1lbnQgYXJlIHN5c3RlbWF0aWNhbGx5IGRpc2N1c3NlZCwgd2hpY2ggaW5jbHVkZXMgaW1wcm92aW5nIHRoZSBhaXIgZmxvdGF0aW9uIHByb2Nlc3MsIGluY3JlYXNpbmcgd2F0ZXIgYWVyYXRpb24gdG8gcHJvbW90ZSBhZXJvYmljIGJpb2xvZ2ljYWwgdGVjaG5vbG9naWVzIGluY2x1ZGluZyBiaW9sb2dpY2FsIGFjdGl2YXRlZCBjYXJib24sIGFjdGl2YXRlZCBzbHVkZ2UsIGFuZCBtZW1icmFuZSBiaW9yZWFjdG9ycywgYW5kIGdlbmVyYXRpbmcgYWN0aXZlIGZyZWUgcmFkaWNhbHMgdGhhdCBveGlkaXNlIG9yZ2FuaWMgY29tcG91bmRzLiBGaW5hbGx5LCB0aGUgY3VycmVudCB0ZWNobm9sb2dpY2FsIGRpZmZpY3VsdGllcyBvZiBidWxrIG5hbm9idWJibGVzIGFyZSBhbmFseXNlZCwgYW5kIGZ1dHVyZSBmb2N1cyBhcmVhcyBmb3IgcmVzZWFyY2ggb24gYnVsayBuYW5vYnViYmxlIHRlY2hub2xvZ3kgYXJlIGFsc28gcHJvcG9zZWQuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjExODYvczEzNTY4LTAyMS0wMTI1NC0wIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2FtYi1leHByZXNzLnNwcmluZ2Vyb3Blbi5jb20vY291bnRlci9wZGYvMTAuMTE4Ni9zMTM1NjgtMDIxLTAxMjU0LTAiLCJpc3N1ZWREYXRlIjoiMjAyMS0wNi0yOCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkFNQiBFeHByZXNzIiwiaXNzdWUiOjEsInZvbHVtZSI6MTEsImltcGFjdEZhY3RvciI6IjQuMjYifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiU3ByaW5nZXIgTmF0dXJlIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo3NiwiY2l0ZWRCeUNvdW50Ijo3NX19LCJxdW90ZSI6IkR1cmluZyB0aGUgc2hyaW5rYWdlIG9mIG5hbm9idWJibGVzLCB0aGUgZWxlYy10cmljIGNoYXJnZSBkZW5zaXR5IGFjY3VtdWxhdGVkIGluIHRoZSBlbGVjdHJpYyBkb3VibGUgIGxheWVyIGluY3JlYXNlcyByYXBpZGx5LiBCZWNhdXNlIG9mIHRoZSBkcmFzdGljIGNoYW5nZSBpbiAgdGhlIGdhc%2BKAk2xpcXVpZCBpbnRlcmZhY2UgYXQgdGhlIG1vbWVudCBvZiBidWJibGUgY29sLWxhcHNlLCB0aGUgaGlnaCBjb25jZW50cmF0aW9uIG9mIGlvbnMgYWdncmVnYXRlZCBvbiB0aGUgIGludGVyZmFjZSBpbnN0YW50bHkgcmVsZWFzZXMgdGhlIGFjY3VtdWxhdGVkIGNoZW1pLWNhbCBlbmVyZ3kuIE1lYW53aGlsZSB0aGUgZW50cmFwcGVkIHdhdGVyIG1vbGVjdWxlcyAgYXJlIHN1YmplY3RlZCB0byBleHRyZW1lbHkgaGlnaCB0ZW1wZXJhdHVyZXMgYW5kIHByZXMtc3VyZXMgd2hlbiB0aGV5IGFyZSBmb3JjZWZ1bGx5IGNvbXByZXNzZWQgYnkgZHluYW1pYyAgc3RpbXVsaSAoS3Jpc2huYW4gZXQgYWwuIDIwMDYpLCB3aGljaCByZXN1bHRzIGluIGNsZWF2YWdlICB0aGF0IHByb2R1Y2VzIHZhcmlvdXMgcmVhY3RpdmUgb3h5Z2VuIHNwZWNpZXMgKFJPU3MpLCAgaW5jbHVkaW5nIGh5ZHJveHlsIHJhZGljYWxzICjCt09IKSwgc3VwZXJveGlkZSBhbmlvbiAgcmFkaWNhbHMgKE8ywrfiiJIpLCBhbmQgc2luZ2xldCBveHlnZW4gKDFPMikgKEF0a2luc29uIGV0IGFsLiAgMjAxOTsgTGl1IGV0IGFsLiAyMDE1OyBMeXUgZXQgYWwuIDIwMTkpLiBTZXZlcmFsIHN0dWRpZXMgaGF2ZSAgYXBwbGllZCBlbGVjdHJvbiBzcGluIHJlc29uYW5jZSBvciB0aGUgYWN0aW9uIG9mIGZyZWUgcmFkaS1jYWwgaW5oaWJpdG9ycyB0byB2ZXJpZnkgdGhlaXIgZXhpc3RlbmNlLiBNaWNoYWlsaWRpIGV0IGFsLiAgKDIwMjApIHVzZWQgZWxlY3Ryb24gcGFyYW1hZ25ldGljIHJlc29uYW5jZSBiYXNlZCBvbiAgNSw1LWRpbWV0aHlsLTEtcHlycm9saW5lIE4tb3hpZGUgKERNUE8pIHNwaW4gdHJhcHBpbmcgIHRvIHF1YW50aXRhdGl2ZWx5IGFuYWx5c2UgZnJlZSByYWRpY2FscyBpbiBhaXItbmFub2J1YmJsZSAgYW5kIG94eWdlbi1uYW5vYnViYmxlIHN5c3RlbXMuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjozLCJiYm94Ijp7InRvcCI6MzUuNTQyMTY3NjYzNTc0MjIsImxlZnQiOjkuNTIzODAwODQ5OTE0NTUsIndpZHRoIjozOS4yODU3MDkzODExMDM1MTYsImhlaWdodCI6MS44Mzk0NzU4NzAxMzI0NDYzfX0seyJwYWdlTnVtYmVyIjozLCJiYm94Ijp7InRvcCI6MzcuMDYxMzA5ODE0NDUzMTI1LCJsZWZ0Ijo5LjUyMzgwMDg0OTkxNDU1LCJ3aWR0aCI6MzkuNjcyOTE2NDEyMzUzNTE2LCJoZWlnaHQiOjEuODM3NjU0NTkwNjA2Njg5NX19LHsicGFnZU51bWJlciI6MywiYmJveCI6eyJ0b3AiOjM4LjU3ODAyNTgxNzg3MTA5NCwibGVmdCI6OS41MjM4MDA4NDk5MTQ1NSwid2lkdGgiOjM5LjY3MzkxMjA0ODMzOTg0NCwiaGVpZ2h0IjoxLjgzNzY1NDU5MDYwNjY4OTV9fSx7InBhZ2VOdW1iZXIiOjMsImJib3giOnsidG9wIjo0MC4wOTQ3NDk0NTA2ODM1OTQsImxlZnQiOjkuNTIzODAwODQ5OTE0NTUsIndpZHRoIjozOS4yODU3MDkzODExMDM1MTYsImhlaWdodCI6MS44Mzg4NjYyMzM4MjU2ODM2fX0seyJwYWdlTnVtYmVyIjozLCJiYm94Ijp7InRvcCI6NDEuNjEzMjg1MDY0Njk3MjY2LCJsZWZ0Ijo5LjUyMzgwMDg0OTkxNDU1LCJ3aWR0aCI6MzkuNjczNDE2MTM3Njk1MzEsImhlaWdodCI6MS44Mzc2NTQ1OTA2MDY2ODk1fX0seyJwYWdlTnVtYmVyIjozLCJiYm94Ijp7InRvcCI6NDMuMTMwMDAxMDY4MTE1MjM0LCJsZWZ0Ijo5LjUyMzgwMDg0OTkxNDU1LCJ3aWR0aCI6MzkuMjg1NzA5MzgxMTAzNTE2LCJoZWlnaHQiOjEuODM4MjYwNTMxNDI1NDc2fX0seyJwYWdlTnVtYmVyIjozLCJiYm94Ijp7InRvcCI6NDQuNjQ3OTMzOTU5OTYwOTQsImxlZnQiOjkuNTIzODAwODQ5OTE0NTUsIndpZHRoIjozOS42NzM1Njg3MjU1ODU5NCwiaGVpZ2h0IjoxLjgzNzY1NDU5MDYwNjY4OTV9fSx7InBhZ2VOdW1iZXIiOjMsImJib3giOnsidG9wIjo0Ni4xNjQ2NDk5NjMzNzg5MDYsImxlZnQiOjkuNTIzODAwODQ5OTE0NTUsIndpZHRoIjozOS4yODU3MDkzODExMDM1MTYsImhlaWdodCI6MS44MzgyNjA1MzE0MjU0NzZ9fSx7InBhZ2VOdW1iZXIiOjMsImJib3giOnsidG9wIjo0Ny42ODI1NzkwNDA1MjczNDQsImxlZnQiOjkuNTIzODAwODQ5OTE0NTUsIndpZHRoIjozOS42NzcyMDQxMzIwODAwOCwiaGVpZ2h0IjoxLjgzNzY1NDU5MDYwNjY4OTV9fSx7InBhZ2VOdW1iZXIiOjMsImJib3giOnsidG9wIjo0OS4xOTkyOTg4NTg2NDI1OCwibGVmdCI6OS41MjM4MDA4NDk5MTQ1NSwid2lkdGgiOjM5LjY2OTQ2NDExMTMyODEyNSwiaGVpZ2h0IjoxLjgzNzY1NDU5MDYwNjY4OTV9fSx7InBhZ2VOdW1iZXIiOjMsImJib3giOnsidG9wIjo1MC43MTYwMTEwNDczNjMyOCwibGVmdCI6OS41MjM4MDA4NDk5MTQ1NSwid2lkdGgiOjM5LjY3NTU1NjE4Mjg2MTMzLCJoZWlnaHQiOjEuODM3NjU0NTkwNjA2Njg5NX19LHsicGFnZU51bWJlciI6MywiYmJveCI6eyJ0b3AiOjUyLjIzMjczMDg2NTQ3ODUxNiwibGVmdCI6OS41MjM4MDA4NDk5MTQ1NSwid2lkdGgiOjM5LjY3MzA4MDQ0NDMzNTk0LCJoZWlnaHQiOjIuMzU5MzM4OTk4Nzk0NTU1N319LHsicGFnZU51bWJlciI6MywiYmJveCI6eyJ0b3AiOjUzLjYwOTM3ODgxNDY5NzI2NiwibGVmdCI6OS41MjM4MDA4NDk5MTQ1NSwid2lkdGgiOjM5LjY3Mjg4MjA4MDA3ODEyNSwiaGVpZ2h0IjoyLjUwMTgzODkyMjUwMDYxMDR9fSx7InBhZ2VOdW1iZXIiOjMsImJib3giOnsidG9wIjo1NS4yNjg1OTI4MzQ0NzI2NTYsImxlZnQiOjkuNTI0NTkwNDkyMjQ4NTM1LCJ3aWR0aCI6MzkuNjY5NDY0MTExMzI4MTI1LCJoZWlnaHQiOjEuODM3NjU0NTkwNjA2Njg5NX19LHsicGFnZU51bWJlciI6MywiYmJveCI6eyJ0b3AiOjU2Ljc4NTMwODgzNzg5MDYyNSwibGVmdCI6OS41MjQ1OTA0OTIyNDg1MzUsIndpZHRoIjozOS4yODQ5MTk3Mzg3Njk1MywiaGVpZ2h0IjoxLjgzODg2NjIzMzgyNTY4MzZ9fSx7InBhZ2VOdW1iZXIiOjMsImJib3giOnsidG9wIjo1OC4zMDM4NDQ0NTE5MDQzLCJsZWZ0Ijo5LjUyMzgwMDg0OTkxNDU1LCJ3aWR0aCI6MzkuNjcwOTU5NDcyNjU2MjUsImhlaWdodCI6MS44Mzc2NTQ1OTA2MDY2ODk1fX0seyJwYWdlTnVtYmVyIjozLCJiYm94Ijp7InRvcCI6NTkuODIwNTY0MjcwMDE5NTMsImxlZnQiOjkuNTIzODAwODQ5OTE0NTUsIndpZHRoIjozOS42NzUzOTIxNTA4Nzg5MDYsImhlaWdodCI6MS44Mzc2NTQ1OTA2MDY2ODk1fX0seyJwYWdlTnVtYmVyIjozLCJiYm94Ijp7InRvcCI6NjEuMzM3MjgwMjczNDM3NSwibGVmdCI6OS41MjM4MDA4NDk5MTQ1NSwid2lkdGgiOjM5LjY3MDQ1MjExNzkxOTkyLCJoZWlnaHQiOjEuODM3NjU0NTkwNjA2Njg5NX19LHsicGFnZU51bWJlciI6MywiYmJveCI6eyJ0b3AiOjYyLjg1NDAwMzkwNjI1LCJsZWZ0Ijo5LjUyMzgwMDg0OTkxNDU1LCJ3aWR0aCI6MzkuNjcwNDM2ODU5MTMwODYsImhlaWdodCI6MS44Mzc2NTQ1OTA2MDY2ODk1fX0seyJwYWdlTnVtYmVyIjozLCJiYm94Ijp7InRvcCI6NjQuMzcwNzEyMjgwMjczNDQsImxlZnQiOjkuNTIzODAwODQ5OTE0NTUsIndpZHRoIjozOS42NzA2MDg1MjA1MDc4MSwiaGVpZ2h0IjoxLjgzNzY1NDU5MDYwNjY4OTV9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI0In19LHsib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlN1dHJpc25vIFNhbG9tbyIsImZhbWlseSI6Ikh1dGFnYWx1bmcifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW5kZSBGdWRqYSIsImZhbWlseSI6IlJhZnJ5YW50byJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJXZWkiLCJmYW1pbHkiOiJTdW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTnVyb2NobWEiLCJmYW1pbHkiOiJKdWxpYXNpaCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTcmkiLCJmYW1pbHkiOiJBZGl0aWEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSml6aG91IiwiZmFtaWx5IjoiSmlhbmcifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQXJyYW1lbCIsImZhbWlseSI6IkFycmFtZWwifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSGVybWF3YW4gS3Jlc25vIiwiZmFtaWx5IjoiRGlwb2pvbm8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU3JpIEhhcmphdGkiLCJmYW1pbHkiOiJTdWhhcmRpIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik51cnVsIFRhdWZpcXUiLCJmYW1pbHkiOiJSb2NobWFuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkRlZGR5IiwiZmFtaWx5IjoiS3VybmlhZGkifX1dLCJpZHMiOnsiaXNzbiI6WyIyMjk2LTY2NVgiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzODkvZmVudnMuMjAyMy4xMTU0NzM5Iiwib3BlbmFsZXgiOiJXNDMyODExNDkyMyJ9LCJ0aXRsZSI6IkNvbWJpbmF0aW9uIG9mIG96b25lLWJhc2VkIGFkdmFuY2VkIG94aWRhdGlvbiBwcm9jZXNzIGFuZCBuYW5vYnViYmxlcyBnZW5lcmF0aW9uIHRvd2FyZCB0ZXh0aWxlIHdhc3Rld2F0ZXIgcmVjb3ZlcnkiLCJhYnN0cmFjdCI6IlRoZSBpbnRyaWNhdGUgbmF0dXJlIG9mIHZhcmlvdXMgdGV4dGlsZSBtYW51ZmFjdHVyaW5nIHByb2Nlc3NlcyBpbnRyb2R1Y2VzIGNvbG9yZWQgZHllcywgc3VyZmFjdGFudHMsIGFuZCB0b3hpYyBjaGVtaWNhbHMgdGhhdCBoYXZlIGJlZW4gaGFybWZ1bCB0byBlY29zeXN0ZW1zIGluIHJlY2VudCB5ZWFycy4gSGVyZSwgYSBjb21iaW5hdGlvbiBvem9uZS1iYXNlZCBhZHZhbmNlZCBveGlkYXRpb24gcHJvY2VzcyAoQU9QKSBpcyBjb3VwbGVkIHdpdGggYSBuYW5vYnViYmxlcyBnZW5lcmF0b3IgZm9yIHRoZSBnZW5lcmF0aW9uIG9mIG96b25lIG5hbm9idWJibGVzIChOQikgdXRpbGl6ZWQgdGhlIHNhbWUgdG8gdHJlYXQgdGhlIHByaW1hcnkgZWZmbHVlbnQgYWNxdWlyZWQgZnJvbSB0ZXh0aWxlIHdhc3Rld2F0ZXJzLiBIZXJlIHdlIGZpbmQgc2V2ZXJhbCBrZXkgcGFyYW1ldGVycyBzdWNoIGFzIGNoZW1pY2FsIG94eWdlbiBkZW1hbmQgYW1tb25pYSBjb250ZW50IChOSCAzICksIGFuZCB0b3RhbCBzdXNwZW5kZWQgc29saWRzIGluZGljYXRpbmcgYSBzdWJzdGFudGlhbCByZWNvdmVyeSBpbiB3aGljaCB0aGUgcmVzcGVjdGl2ZSBwZXJjZW50YWdlcyBvZiA4MS4xJSwgMzAuODElLCBhbmQgNDEuOTglLCB1cG9uIDMwMCBtaW4gcmVzaWRlbmNlIHRpbWUgYXJlIGFjaGlldmVkLiBPbiB0aGUgb3RoZXIgaGFuZCwgdGhlIHBIIGlzIHNoaWZ0ZWQgZnJvbSA3LjkzIHRvIDcuNDYsIGluZGljYXRpbmcgdGhlIGdlbmVyYXRpb24gb2YgaHlkcm9nZW4gcGVyb3hpZGUgKEggMiBPIDIgKSBkdWUgdG8gdGhlIHRlcm1pbmF0aW9uIHJlYWN0aW9uIGFuZCB0aGUgc2VsZi1yZWFjdGlvbiBvZiByZWFjdGl2ZSBveHlnZW4gc3BlY2llcyAoUk9TKS4gV2UgcHJvcG9zZSB0aGF0IHRoZSByZWFjdGl2ZSBveHlnZW4gc3BlY2llcyBjYW4gYmUgaWRlbnRpZmllZCBmcm9tIHRoZSBuZWdhdGl2ZSB6ZXRhIHBvdGVudGlhbCBtZWFzdXJlbWVudCAo4oiSMjIuNDMgwrEgMC4zNCBtVikgY29sbGVjdGVkIGluIHRoZSBmaW5hbCBzdGF0ZSBvZiB0cmVhdG1lbnQuIFRoZSBjb21iaW5lZCBtZXRob2QgaGFzIHN1Y2Nlc3NmdWxseSBnZW5lcmF0ZWQgb3pvbmUgbmFub2J1YmJsZXMgd2l0aCA5OS45NCUgb2Ygc2l6ZSBkaXN0cmlidXRlZCBpbiAyMTYuOSBubS4gVGhpcyBoaWdobGlnaHRzIHRoYXQgZW5oYW5jZW1lbnQgb2Ygb3pvbmXigJlzIHJlYWN0aXZpdHkgcGxheXMgYSBjcnVjaWFsIHJvbGUgaW4gaW1wcm92aW5nIHRoZSB3YXRlciBxdWFsaXR5IG9mIHRleHRpbGUgd2FzdGV3YXRlciB0b3dhcmRzIGJlaW5nIHRlY2hub2xvZ2ljYWxseSBlZmZpY2llbnQgdG8gZGF0ZS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMzM4OS9mZW52cy4yMDIzLjExNTQ3MzkiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vd3d3LmZyb250aWVyc2luLm9yZy9hcnRpY2xlcy8xMC4zMzg5L2ZlbnZzLjIwMjMuMTE1NDczOS9wZGYiLCJpc3N1ZWREYXRlIjoiMjAyMy0wMy0yMSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZyb250aWVycyBpbiBFbnZpcm9ubWVudGFsIFNjaWVuY2UiLCJ2b2x1bWUiOjExLCJpbXBhY3RGYWN0b3IiOiI1LjY0In0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkZyb250aWVycyBNZWRpYSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NjQsImNpdGVkQnlDb3VudCI6MTl9fSwicXVvdGUiOiJPdXIgZmluZGluZyByZWdhcmRpbmcgQ09EIHJlZHVjdGlvbiB0aGF0IG9wZXJhdGVkIGF0IG5ldXRyYWwgcEggaW4gYWdyZWVtZW50IHdpdGggdGhlIHByZXZpb3VzIHJlcG9ydHMgKEx1Y2FzIGV0IGFsLiwgMjAwOTsgVGFrYWhhc2hpIGV0IGFsLiwgMjAxMjsgWWFuZyBldCBhbC4sIDIwMTI7IFRlbWVzZ2VuIGV0IGFsLiwgMjAxNykuIFRoZSBzZWNvbmQgcmVjb3Zlcnkgc3RhZ2UgY29ycmVzcG9uZHMgdG8gdGhlIHV0aWxpemF0aW9uIG9mIG96b25lLWJhc2VkIEFPUCBhcyBpbmRpY2F0ZWQgYnkgdGhlIGJsdWUgYXJyb3cgaW4gRmlndXJlIDcuIEluIHRoaXMgc2VjdGlvbiwgdGhlIGdlbmVyYXRpb24gb2YgUk9TIG9yIG5lZ2F0aXZlbHktY2hhcmdlZCBOQiBkb21pbmFudGx5IGNvbnRyaWJ1dGVkIHRvIGRlZ3JhZGUgdGhlIG9yZ2FuaWMgY29udGFtaW5hbnRzLiBXZSByZXZpc2l0IHRoZSBjb252ZW50aW9uYWwgdGhlcm1vZHluYW1pY3Mgb2YgYnViYmxlIGNvbGxhcHNlIHBoZW5vbWVub24gdG8gZGVzY3JpYmUgdGhlIFJPUyBnZW5lcmF0aW9uIChHaGFkaW1raGFuaSBldCBhbC4sIDIwMTYpIGFuZCBvem9uZSBicmVha2Rvd24gKElraGxhcSBldCBhbC4sIDIwMTM7IE9oIGFuZCBOZ3V5ZW4sIDIwMjIpLiBJbiB0aGUgb3pvbmUgZGVjb21wb3NpdGlvbiwgd2UgYXR0cmlidXRlIHRoZSBST1MgaXMgYSBjaGFpbiByZWFjdGlvbiBjb21wcmlzZWQgb2YgaW5pdGlhdGlvbiwgcHJvcGFnYXRpb24sIGFuZCB0ZXJtaW5hdGlvbiAoS2h1bnRpYSBldCBhbC4sIDIwMTUpLiBGdXJ0aGVybW9yZSwgUk9TIHZpYSBjb2xsYXBzaW5nIGNhdml0eSBidWJibGVzIGNhbiBiZSBkZXNjcmliZWQgYmFzZWQgb24gdGhlIFlvdW5nLUxhcGxhY2UgZXF1YXRpb24gKEtpbXVyYSBldCBhbC4sIDIwMDQ7IFRha2FoYXNoaSBldCBhbC4sIDIwMDc7IEdoYWRpbWtoYW5pIGV0IGFsLiwgMjAxNikuIFAg77%2B9IFAxICsgNM%2BD77%2B9ZCAoMykgd2hlcmUgUCBkZW5vdGVzIGdhcyBwcmVzc3VyZSwgUGwgZGVub3RlcyBsaXF1aWQgcHJlc3N1cmUsIM%2BDIGRlbm90ZXMgbGlxdWlkIHN1cmZhY2UgdGVuc2lvbiwgYW5kIGQgZGVub3RlcyB0aGUgYnViYmxlIGRpYW1ldGVyLiBUaGUgaW50ZXJuYWwgcHJlc3N1cmUgb2YgYSBidWJibGUgaXMgc3Vic3RhbnRpYWxseSBkZXRlcm1pbmVkIGJ5IGl0cyBzaXplLCBhY2NvcmRpbmcgdG8gRXEuIDMuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NTkuMjc4ODkyNTE3MDg5ODQ0LCJsZWZ0Ijo4LjMzMzE3Mjc5ODE1NjczOCwid2lkdGgiOjQwLjQ3MTgzNjA5MDA4Nzg5LCJoZWlnaHQiOjEuMTc1NzgzNTE0OTc2NTAxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjYwLjY5OTczNzU0ODgyODEyNSwibGVmdCI6OC4zMzMxNzA4OTA4MDgxMDUsIndpZHRoIjo0MC40NzA5MzIwMDY4MzU5NCwiaGVpZ2h0IjoxLjE3NTc4MzUxNDk3NjUwMTV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo2Mi4xMjA1NzQ5NTExNzE4NzUsImxlZnQiOjguMzMzMTcwODkwODA4MTA1LCJ3aWR0aCI6NDAuNDcxMzgyMTQxMTEzMjgsImhlaWdodCI6MS4xNzU3ODM1MTQ5NzY1MDE1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NjMuNTM0NzE3NTU5ODE0NDUsImxlZnQiOjguMzMzMTcyNzk4MTU2NzM4LCJ3aWR0aCI6MTQuMTAyMTk2NjkzNDIwNDEsImhlaWdodCI6MS4xNzU3ODM1MTQ5NzY1MDE1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NjQuOTU1NTU4Nzc2ODU1NDcsImxlZnQiOjEwLjY3NTk4NDM4MjYyOTM5NSwid2lkdGgiOjM4LjEzMjk0MjE5OTcwNzAzLCJoZWlnaHQiOjEuMTc1NzgzNTE0OTc2NTAxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjY2LjM3NjQwMzgwODU5Mzc1LCJsZWZ0Ijo4LjMzMzE3NTY1OTE3OTY4OCwid2lkdGgiOjQwLjQ3MDkzMjAwNjgzNTk0LCJoZWlnaHQiOjEuMTc1NzgzNTE0OTc2NTAxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjY3Ljc5NzI0ODg0MDMzMjAzLCJsZWZ0Ijo4LjMzMzE3NzU2NjUyODMyLCJ3aWR0aCI6NC41NDQzMzI5ODExMDk2MTksImhlaWdodCI6MS4xNzU3ODM1MTQ5NzY1MDE1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NjcuNzk3MjQ4ODQwMzMyMDMsImxlZnQiOjE0LjE5MDIwMTc1OTMzODM3OSwid2lkdGgiOjEuODgxMjk3NzA3NTU3Njc4MiwiaGVpZ2h0IjoxLjE3NTc4MzUxNDk3NjUwMTV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo2Ny43OTcyNDg4NDAzMzIwMywibGVmdCI6MTcuMzcxMDg2MTIwNjA1NDcsIndpZHRoIjo2LjM3ODkzODE5ODA4OTYsImhlaWdodCI6MS4xNzU3ODM1MTQ5NzY1MDE1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NjcuNzk3MjQ4ODQwMzMyMDMsImxlZnQiOjI1LjA2NjAyNjY4NzYyMjA3LCJ3aWR0aCI6MS4xOTE0MzgzMTcyOTg4ODkyLCJoZWlnaHQiOjEuMTc1NzgzNTE0OTc2NTAxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjY3Ljc5NzI0ODg0MDMzMjAzLCJsZWZ0IjoyNy41NjEyNzE2Njc0ODA0Nywid2lkdGgiOjIuNzQyODY4MTg1MDQzMzM1LCJoZWlnaHQiOjEuMTc1NzgzNTE0OTc2NTAxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjY3Ljc5NzI0ODg0MDMzMjAzLCJsZWZ0IjozMS42MDg4NDY2NjQ0Mjg3MSwid2lkdGgiOjEuMzAxMzk0MjI0MTY2ODcwMSwiaGVpZ2h0IjoxLjE3NTc4MzUxNDk3NjUwMTV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo2Ny43OTcyNDg4NDAzMzIwMywibGVmdCI6MzQuMjE4NDE0MzA2NjQwNjI1LCJ3aWR0aCI6MTEuMjg0NzcxOTE5MjUwNDg4LCJoZWlnaHQiOjEuMTc1NzgzNTE0OTc2NTAxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjY3Ljc5NzI0ODg0MDMzMjAzLCJsZWZ0Ijo0Ni44MTc5ODU1MzQ2Njc5Nywid2lkdGgiOjEuOTkxMjUzNDk1MjE2MzY5NiwiaGVpZ2h0IjoxLjE3NTc4MzUxNDk3NjUwMTV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo2OS4yMTgwOTM4NzIwNzAzMSwibGVmdCI6OC4zMzMxNzc1NjY1MjgzMiwid2lkdGgiOjQwLjQ3Mzc5MzAyOTc4NTE1NiwiaGVpZ2h0IjoxLjE3NTc4MzUxNDk3NjUwMTV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo3MC42Mzg5Mzg5MDM4MDg2LCJsZWZ0Ijo4LjMzMzE3NzU2NjUyODMyLCJ3aWR0aCI6NDAuNDcxOTg4Njc3OTc4NTE2LCJoZWlnaHQiOjEuMTc1NzgzNTE0OTc2NTAxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjcyLjA1OTc3NjMwNjE1MjM0LCJsZWZ0Ijo4LjMzMzE3NzU2NjUyODMyLCJ3aWR0aCI6NDAuNDY2NDExNTkwNTc2MTcsImhlaWdodCI6MS4xNzU3ODM1MTQ5NzY1MDE1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NzMuNDgwNjIxMzM3ODkwNjIsImxlZnQiOjguMzMzMTc3NTY2NTI4MzIsIndpZHRoIjo0MC40NzU1OTczODE1OTE4LCJoZWlnaHQiOjEuMTc1NzgzNTE0OTc2NTAxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjc0Ljg5NDc1MjUwMjQ0MTQsImxlZnQiOjguMzMzMTc3NTY2NTI4MzIsIndpZHRoIjo0MC40NzIyOTM4NTM3NTk3NjYsImhlaWdodCI6MS4xNzU3ODM1MTQ5NzY1MDE1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NzYuMzE1NTk3NTM0MTc5NjksImxlZnQiOjguMzMzMTc4NTIwMjAyNjM3LCJ3aWR0aCI6MC42NTgyMjgwOTkzNDYxNjA5LCJoZWlnaHQiOjEuMTc1NzgzNTE0OTc2NTAxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjc2LjMxNTU5NzUzNDE3OTY5LCJsZWZ0IjoxMC4xOTk4NjUzNDExODY1MjMsIndpZHRoIjoxNi44NTUxNTU5NDQ4MjQyMiwiaGVpZ2h0IjoxLjE3NTc4MzUxNDk3NjUwMTV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo3Ni4zMTU1OTc1MzQxNzk2OSwibGVmdCI6MjguMjY2MDM4ODk0NjUzMzIsIndpZHRoIjoxLjE5MTQzODMxNzI5ODg4OTIsImhlaWdodCI6MS4xNzU3ODM1MTQ5NzY1MDE1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NzYuMzE1NTk3NTM0MTc5NjksImxlZnQiOjMwLjY2NTkzNzQyMzcwNjA1NSwid2lkdGgiOjUuODQ1NzI3OTIwNTMyMjI3LCJoZWlnaHQiOjEuMTc1NzgzNTE0OTc2NTAxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjc2LjMxNTU5NzUzNDE3OTY5LCJsZWZ0IjozNy43MjI4MzkzNTU0Njg3NSwid2lkdGgiOjcuNjE3MDY5NzIxMjIxOTI0LCJoZWlnaHQiOjEuMTc1NzgzNTE0OTc2NTAxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjc2LjMxNTU5NzUzNDE3OTY5LCJsZWZ0Ijo0Ni41NTEwNzg3OTYzODY3Miwid2lkdGgiOjIuMjU2MzUxMjMyNTI4Njg2NSwiaGVpZ2h0IjoxLjE3NTc4MzUxNDk3NjUwMTV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo3Ny43MzY0NTAxOTUzMTI1LCJsZWZ0Ijo4LjMzMzE3ODUyMDIwMjYzNywid2lkdGgiOjIxLjA4MzAyNjg4NTk4NjMyOCwiaGVpZ2h0IjoxLjE3NTc4MzUxNDk3NjUwMTV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo3OS4xNTcyOTUyMjcwNTA3OCwibGVmdCI6MTAuNjc1OTg3MjQzNjUyMzQ0LCJ3aWR0aCI6MzguMTI5OTIwOTU5NDcyNjU2LCJoZWlnaHQiOjEuMTc1NzgzNTE0OTc2NTAxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjgwLjU3ODE0MDI1ODc4OTA2LCJsZWZ0Ijo4LjMzMzAyNzgzOTY2MDY0NSwid2lkdGgiOjQwLjQ3Nzg1NTY4MjM3MzA1LCJoZWlnaHQiOjEuMTc1NzgzNTE0OTc2NTAxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjgxLjk5ODk3NzY2MTEzMjgxLCJsZWZ0Ijo4LjMzMzAyNjg4NTk4NjMyOCwid2lkdGgiOjM4LjE0NzU0NDg2MDgzOTg0NCwiaGVpZ2h0IjoxLjE3NTc4MzUxNDk3NjUwMTV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo4My4xMDEzNDEyNDc1NTg2LCJsZWZ0IjoyNC4yMDkyNzA0NzcyOTQ5MjIsIndpZHRoIjoyNC42MDI1OTI0NjgyNjE3MiwiaGVpZ2h0Ijo1LjM0NjQwNjQ1OTgwODM1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6ODYuMTgxMzgxMjI1NTg1OTQsImxlZnQiOjguMzMzMjU0ODE0MTQ3OTUsIndpZHRoIjo0MC40NzQ0MTQ4MjU0Mzk0NSwiaGVpZ2h0IjoxLjM1NzYwNzk2MDcwMDk4ODh9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo4Ny42MDIyMjYyNTczMjQyMiwibGVmdCI6OC4zMzMyNzc3MDIzMzE1NDMsIndpZHRoIjo0MC40NzQ1NDQ1MjUxNDY0ODQsImhlaWdodCI6MS4xNzU3ODM1MTQ5NzY1MDE1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6ODkuMDIzMDc4OTE4NDU3MDMsImxlZnQiOjguMzMzNDMwMjkwMjIyMTY4LCJ3aWR0aCI6NDAuNDcyNzQwMTczMzM5ODQ0LCJoZWlnaHQiOjEuMTc1NzgzNTE0OTc2NTAxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjkwLjQ0MzkyMzk1MDE5NTMxLCJsZWZ0Ijo4LjMzMzQzMDI5MDIyMjE2OCwid2lkdGgiOjQwLjQ3NjgwNjY0MDYyNSwiaGVpZ2h0IjoxLjE3NTc4MzUxNDk3NjUwMTV9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI2In19XQ%3D%3D. These ROS oxidize microbial cell walls, proteins, and nucleic acids, while negative zeta potentials enhance pathogen adsorption via electrostatic and hydrophobic interactions (Jia et al., 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjMwLCJzdWJ0eXBlIjp7fSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTWluZ3lpIiwiZmFtaWx5IjoiSmlhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik11aGFtbWFkIFVzbWFuIiwiZmFtaWx5IjoiRmFyaWQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSmVoYWQgQS4iLCJmYW1pbHkiOiJLaGFycmF6In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hbGxhcGFuZW5pIE1hbm9qIiwiZmFtaWx5IjoiS3VtYXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2hhdWhyYXQgUy4iLCJmYW1pbHkiOiJDaG9wcmEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW0iLCJmYW1pbHkiOiJKYW5nIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlkuTS4gSm9obiIsImZhbWlseSI6IkNoZXcifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2FtaXIgS3VtYXIiLCJmYW1pbHkiOiJLaGFuYWwifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiR3VhbmdoYW8iLCJmYW1pbHkiOiJDaGVuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFsaWNpYSBLeW91bmdqaW4iLCJmYW1pbHkiOiJBbiJ9fV0sImlkcyI6eyJpc3NuIjpbIjAwNDMtMTM1NCIsIjE4NzktMjQ0OCJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMTAxNi9qLndhdHJlcy4yMDIzLjEyMDYxMyIsInBtaWQiOiIzNzczODk0MCIsIm9wZW5hbGV4IjoiVzQzODY2MTg0NTkifSwidGl0bGUiOiJOYW5vYnViYmxlcyBpbiB3YXRlciBhbmQgd2FzdGV3YXRlciB0cmVhdG1lbnQgc3lzdGVtczogU21hbGwgYnViYmxlcyBtYWtpbmcgYmlnIGRpZmZlcmVuY2UiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAxNi9qLndhdHJlcy4yMDIzLjEyMDYxMyIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9wdXJlaG9zdC5iYXRoLmFjLnVrL3dzL2ZpbGVzLzI5NjM0MzkwMS9OYW5vYnViYmxlc19wcmVfcHJvb2YucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjMtMDktMTEiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA4IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJXYXRlciBSZXNlYXJjaCIsInZvbHVtZSI6MjQ1LCJpbXBhY3RGYWN0b3IiOiIxMy40NCJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjEyMDYxMywibGFzdFBhZ2UiOjEyMDYxM319LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJFbHNldmllciBCViJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MTExLCJjaXRlZEJ5Q291bnQiOjc1fX0sInF1b3RlIjoiU2V2ZXJhbCBzdHVkaWVzIGhhdmUgcHJvcG9zZWQgdGhhdCB0aGUgbmVnYXRpdmUgY2hhcmdlIG9mIE5CcyBhcmlzZXMgZHVlIHRvIHRoZWlyIHNtYWxsIHJhZGl1cyBhbmQgdGhlIGhpZ2ggcG9sYXJpdHkgb2YgaHlkcm94eWwgaW9ucywgd2hpY2ggbGVhZCB0byB0aGUgYWRzb3JwdGlvbiBvZiBkaXNzb2NpYXRlZCBoeWRyb3h5bCBpb25zIG9uIHRoZSBidWJibGUgc3VyZmFjZSwgd2hpbGUgaHlkcm9nZW4gaW9ucyBhcmUgZWFzaWx5IGh5ZHJhdGVkIGFuZCBwcmVmZXIgdG8gc3RheSBpbiB0aGUgYnVsayBsaXF1aWQuIEEgaGlnaGVyIGFic29sdXRlIHpldGEgcG90ZW50aWFsIGluZGljYXRlcyBoaWdoZXIgc3RhYmlsaXR5LCBhcyB0aGUgcmVwdWxzaXZlIGZvcmNlIGdlbmVyYXRlZCBieSBOQnPigJggc3VyZmFjZXMgcHJldmVudHMgdGhlbSBmcm9tIG1lcmdpbmcgd2l0aCBvbmUgYW5vdGhlciAoVXNoaWt1Ym8gZXQgYWwuLCAyMDEwKS4gMTEgU3VyZmFjZSBoeWRyb3Bob2JpY2l0eSBpcyBhbm90aGVyIGVzc2VudGlhbCBwcm9wZXJ0eSBvZiBOQnMgYXMgaXQgYWxsb3dzIE5CcyB0byBpbnRlcmFjdCB3aXRoIG90aGVyIHBhcnRpY2xlcyB0aHJvdWdoIGh5ZHJvcGhvYmljIGF0dHJhY3Rpb24uIEJOQnMgY2FuIGFkc29yYiBvbiBoeWRyb3Bob2JpYyBwYXJ0aWNsZXMgYW5kIGluY3JlYXNlIHRoZWlyIG1vYmlsaXR5IGluIHRoZSBsaXF1aWQsIHdoaWxlIGF0dGFjaGVkIE5CcyBjYW4gcHJvbW90ZSBwYXJ0aWNsZS1wYXJ0aWNsZSBvciBwYXJ0aWNsZS0gYnViYmxlIGludGVyYWN0aW9ucyB0aHJvdWdoIGNhcGlsbGFyeSBicmlkZ2luZyBlZmZlY3RzIHRoYXQgcHJvZHVjZSBhbiBhdHRyYWN0aXZlIGZvcmNlIGJldHdlZW4gdHdvIHN1cmZhY2VzIChIYW1wdG9uIGFuZCBOZ3V5ZW4sIDIwMTApLiAyLjEuMyBMb25nIHJlc2lkZW5jZSB0aW1lIGluIHdhdGVyOiBOQnPigJkgbG9uZy10ZXJtIHN0YWJpbGl0eSBBbW9uZyB0aGUgdmFyaW91cyBjaGFyYWN0ZXJpc3RpY3MgTkJzIHBvc3Nlc3MsIHRoZSBtb3N0IGFzdG9uaXNoaW5nIGlzIHRoZWlyIGxvbmdldml0eS4gVXBvbiBnZW5lcmF0aW9uLCBidWJibGVzIGdlbmVyYWxseSBtYWludGFpbiB0aGVpciBkaWFtZXRlcnMgZm9yIGEgY2VydGFpbiBhbW91bnQgb2YgdGltZSwgYWZ0ZXIgd2hpY2ggdGhlIGJ1YmJsZXMgYmVnaW4gdG8gZ3JvdyBhbmQgZXZlbnR1YWxseSBkaXNzb2x2ZSBpbiB0aGUgc29sdXRpb24uIEV4cGVyaW1lbnRhbCByZXN1bHRzIGhhdmUgY29uZmlybWVkIHRoYXQgTkJzIGFyZSBhYmxlIHRvIGV4aXN0IGluIHdhdGVyIGZyb20gZGF5cyB0byBtb250aHMuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxMSwiYmJveCI6eyJ0b3AiOjY4LjYzMTk2NTYzNzIwNzAzLCJsZWZ0Ijo5Ljc0MTE4MDQxOTkyMTg3NSwid2lkdGgiOjc3LjAxMDI1MzkwNjI1LCJoZWlnaHQiOjEuODQ5MjYzNjY4MDYwMzAyN319LHsicGFnZU51bWJlciI6MTEsImJib3giOnsidG9wIjo3MS44MjM4MjIwMjE0ODQzOCwibGVmdCI6OS43NDExODA0MTk5MjE4NzUsIndpZHRoIjo3Ny4wMDYzMDk1MDkyNzczNCwiaGVpZ2h0IjoxLjg0OTI2MzY2ODA2MDMwMjd9fSx7InBhZ2VOdW1iZXIiOjExLCJiYm94Ijp7InRvcCI6NzUuMDEyMjE0NjYwNjQ0NTMsImxlZnQiOjkuNzQxMTgwNDE5OTIxODc1LCJ3aWR0aCI6NzYuOTQ4Nzc2MjQ1MTE3MTksImhlaWdodCI6MS44NDkyNjM2NjgwNjAzMDI3fX0seyJwYWdlTnVtYmVyIjoxMSwiYmJveCI6eyJ0b3AiOjc4LjIwMDU5OTY3MDQxMDE2LCJsZWZ0Ijo5Ljc0MTE4MDQxOTkyMTg3NSwid2lkdGgiOjc3LjAwODI4NTUyMjQ2MDk0LCJoZWlnaHQiOjEuODQ5MjYzNjY4MDYwMzAyN319LHsicGFnZU51bWJlciI6MTEsImJib3giOnsidG9wIjo4MS4zODg5OTk5Mzg5NjQ4NCwibGVmdCI6OS43NDExODA0MTk5MjE4NzUsIndpZHRoIjo3Ny4wMTAyNDYyNzY4NTU0NywiaGVpZ2h0IjoxLjg0OTI2MzY2ODA2MDMwMjd9fSx7InBhZ2VOdW1iZXIiOjExLCJiYm94Ijp7InRvcCI6ODQuNTc2OTE5NTU1NjY0MDYsImxlZnQiOjkuNzQxMTgwNDE5OTIxODc1LCJ3aWR0aCI6MTkuNjE2NjI0ODMyMTUzMzIsImhlaWdodCI6MS44NDkyNjM2NjgwNjAzMDI3fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjg3LjQ4ODA1OTk5NzU1ODYsImxlZnQiOjQ3LjAxODI1MzMyNjQxNjAxNiwid2lkdGgiOjIuNDUyMTYxNzg4OTQwNDI5NywiaGVpZ2h0IjoxLjg0OTI2MzY2ODA2MDMwMjd9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MTEuNjQ4NzM4ODYxMDgzOTg0LCJsZWZ0Ijo5Ljc0MTE4MDQxOTkyMTg3NSwid2lkdGgiOjc2Ljk1MjcwNTM4MzMwMDc4LCJoZWlnaHQiOjEuODQ5MjY1NTc1NDA4OTM1NX19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoxNC44Mzk0MzQ2MjM3MTgyNjIsImxlZnQiOjkuNzQxMTgwNDE5OTIxODc1LCJ3aWR0aCI6NzYuOTQ4NzgzODc0NTExNzIsImhlaWdodCI6MS44NDkyNjU1NzU0MDg5MzU1fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjE4LjAyNzgzMzkzODU5ODYzMywibGVmdCI6OS43NDExODA0MTk5MjE4NzUsIndpZHRoIjo3Ni41MDYwNzI5OTgwNDY4OCwiaGVpZ2h0IjoxLjg0OTI2NTU3NTQwODkzNTV9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjEuMjE2MjE3MDQxMDE1NjI1LCJsZWZ0Ijo5Ljc0MTE4MDQxOTkyMTg3NSwid2lkdGgiOjc2Ljk4NTczMzAzMjIyNjU2LCJoZWlnaHQiOjEuODQ5MjY1NTc1NDA4OTM1NX19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyNC40MDQ2MDAxNDM0MzI2MTcsImxlZnQiOjkuNzQxMTgwNDE5OTIxODc1LCJ3aWR0aCI6MzUuMzc0MTk4OTEzNTc0MjIsImhlaWdodCI6MS44NDkyNjU1NzU0MDg5MzU1fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjMyLjIxMTMxMTM0MDMzMjAzLCJsZWZ0Ijo5Ljc0MTE4MDQxOTkyMTg3NSwid2lkdGgiOjUyLjQ2NDEyMjc3MjIxNjgsImhlaWdodCI6MS45MTk5NjUyNjcxODEzOTY1fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM2LjA1MjYyNzU2MzQ3NjU2LCJsZWZ0Ijo5Ljc0MTE4MDQxOTkyMTg3NSwid2lkdGgiOjc3LjAwODI4NTUyMjQ2MDk0LCJoZWlnaHQiOjEuODQ5MjY3NDgyNzU3NTY4NH19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozOS4yNDEwMTYzODc5Mzk0NSwibGVmdCI6OS43NDExODA0MTk5MjE4NzUsIndpZHRoIjo3Ny4wMDgyODU1MjI0NjA5NCwiaGVpZ2h0IjoxLjg0OTI2NzQ4Mjc1NzU2ODR9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDIuNDI5NDA5MDI3MDk5NjEsImxlZnQiOjkuNzQxMTgwNDE5OTIxODc1LCJ3aWR0aCI6NzcuMDAyNDAzMjU5Mjc3MzQsImhlaWdodCI6MS44NDkyNjc0ODI3NTc1Njg0fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQ1LjYxNzc5NDAzNjg2NTIzNCwibGVmdCI6OS43NDExODA0MTk5MjE4NzUsIndpZHRoIjo3Ny4wMTAyNDYyNzY4NTU0NywiaGVpZ2h0IjoxLjg0OTI2NzQ4Mjc1NzU2ODR9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIxMjA2MjQifX1d. This multifaceted approach suppresses biofilms and planktonic cells alike, offering a sustainable alternative for biosecurity.

Empirical studies underscore nanobubbles' efficacy against aquaculture-specific pathogens. In lab-scale trials with Pacific white shrimp, oxygen nanobubbles generated by the aQua+75MO system significantly reduced Vibrio parahaemolyticus loads, correlating with improved survival rates and water quality (Nghĩa et al., 2022)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5ndXnhu4VuIEjhu691IiwiZmFtaWx5IjoiTmdoxKlhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5ndXllbiBUaGkiLCJmYW1pbHkiOiJOZ3V5ZW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUGhhbiBUcuG7jW5nIiwiZmFtaWx5IjoiQsOsbmgifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTGUgVGhpIiwiZmFtaWx5IjoiTWF5In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRvbmcgVHJhbiIsImZhbWlseSI6Ikh1eSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQaOG6oW0gVGjDoWkiLCJmYW1pbHkiOiJHaWFuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTb3BoaWUiLCJmYW1pbHkiOiJTdOKAkEhpbGFpcmUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUGhhbiBUaGkiLCJmYW1pbHkiOiJWYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMjM0LTE3NDkiLCIyMjM0LTE3NTciXSwiaXNibiI6W10sImRvaSI6IjEwLjQ3ODUzL2Zhcy4yMDIyLmUzOSIsIm9wZW5hbGV4IjoiVzQyOTQ1OTc1NTQifSwidGl0bGUiOiJFZmZlY3Qgb2YgbmFub2J1YmJsZXMgKG94eWdlbiwgb3pvbmUpIG9uIHRoZSBQYWNpZmljIHdoaXRlIHNocmltcCAoUGVuYWV1cyB2YW5uYW1laSksIFZpYnJpbyBwYXJhaGFlbW9seXRpY3VzIGFuZCB3YXRlciBxdWFsaXR5IHVuZGVyIGxhYiBjb25kaXRpb25zIiwiYWJzdHJhY3QiOiJUaGlzIHN0dWR5IGFzc2Vzc2VkIHRoZSBlZmZlY3RzIG9mIG94eWdlbiBhbmQgb3pvbmUgbmFub2J1YmJsZXMgb24gZ2lsbCBtb3JwaG9sb2d5LCB3ZWlnaHQgZ2FpbiwgYW5kIG1vcnRhbGl0eSBvZiBQYWNpZmljIHdoaXRlIHNocmltcCAoUGVuYWV1cyB2YW5uYW1laSksIGFzIHdlbGwgYXMgdGhlIGxldmVsIG9mIFZpYnJpbyBwYXJhaGFlbW9seXRpY3VzIGFuZCB3YXRlciBxdWFsaXR5IG9mIHNocmltcCBjdWx0dXJlIHRhbmtzIHVuZGVyIGxhYiBjb25kaXRpb25zLiBUd28gZXhwZXJpbWVudHMgd2VyZSBjYXJyaWVkIG91dCB3aXRoIG94eWdlbiBtYWNyb2J1YmJsZSwgb3pvbmUgbWFjcm9idWJibGUsIG94eWdlbiBuYW5vYnViYmxlLCBvem9uZSBuYW5vYnViYmxlLCBhbmQgY29udHJvbCB0cmVhdG1lbnRzIChhaXItc3RvbmUgbWFjcm9idWJibGUpLiBFeHBlcmltZW50cyB3ZXJlIGRvbmUgaW4gdHJpcGxpY2F0ZSBpbiAxMDAgTCB0YW5rcyB3aXRoIDE14oCwIHNhbGluZSB3YXRlciwgYW5kIDIwIHNocmltcCBwZXIgdGFuay4gVGFua3MgaW4gRXhwZXJpbWVudCAxIHdlcmUgbm90IGlub2N1bGF0ZWQgd2l0aCBiYWN0ZXJpYTsgdGFua3MgaW4gRXhwZXJpbWVudCAyIHdlcmUgaW5vY3VsYXRlZCB3aXRoIFYuIHBhcmFoYWVtb2x5dGljdXMgYXQgYSBjb25jZW50cmF0aW9uIG9mIDEwNiBDRlUvbUwuIFRoZSByZXN1bHRzIHJldmVhbGVkIHRoYXQgc2hvcnQgdHJlYXRtZW50cyB3aXRoIG96b25lIG5hbm9idWJibGVzIGhhZCBtaW5pbWFsIGltcGFjdCBvbiBzaHJpbXAgZ2lsbHMsIG1vcnRhbGl0eSwgYW5kIGdyb3d0aCByYXRlcywgcmVkdWNlZCBWLiBwYXJhaGFlbW9seXRpY3VzIGNvbmNlbnRyYXRpb24gaW4gd2F0ZXIgY29tcGFyZWQgdG8gdGhlIG90aGVyIGdyb3VwcywgYW5kIGltcHJvdmUgd2F0ZXIgcXVhbGl0eS4gVGhlc2UgbGFib3JhdG9yeSByZXN1bHRzIGluZGljYXRlIHRoYXQgb3pvbmUgbmFub2J1YmJsZSB0cmVhdG1lbnQgbWF5IGJlIHVzZWZ1bCBmb3IgY29udHJvbGxpbmcgVi4gcGFyYWhhZW1vbHl0aWN1cy4gTW9yZSB3b3JrIGlzIG5lZWRlZCB0byBmaW5kIHRoZSBiZXN0IHByb3RvY29sIHRvIGFwcGx5IHRoZSB0ZWNobm9sb2d5IG9uIGEgY29tbWVyY2lhbCBzY2FsZS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuNDc4NTMvZmFzLjIwMjIuZTM5IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5lLWZhcy5vcmcvZG93bmxvYWQvZG93bmxvYWRfcGRmP3BpZD1mYXMtMjUtOC00MjkiLCJpc3N1ZWREYXRlIjoiMjAyMi0wOC0wMSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZpc2hlcmllcyBhbmQgYXF1YXRpYyBzY2llbmNlcyIsImlzc3VlIjo4LCJ2b2x1bWUiOjI1LCJpbXBhY3RGYWN0b3IiOiIxLjM0In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6NDI5LCJsYXN0UGFnZSI6NDQwfX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlNwcmluZ2VyIFNjaWVuY2UrQnVzaW5lc3MgTWVkaWEifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjMwLCJjaXRlZEJ5Q291bnQiOjEzfX0sInF1b3RlIjoiQSBuYW5vYnViYmxlIGdlbmVyYXRvciBtb2RlbCBhUXVhKzc1TU8gKEFxdWFQcm8gU29sdXRpb25zLCBTaW5nYXBvcmUsIFNpbmdhcG9yZSkgd2l0aCBhIDEgSFAgcG93ZXIgKDAuNzUga1cpIGh0dHBzOi8vZG9pLm9yZy8xMC40Nzg1My9GQVMuMjAyMi5lMzkgaHR0cHM6Ly93d3cuZS1mYXMub3JnIHwgNDMxIE5ndXllbiBIdXUgTmdoaWEsIGV0IGFsLiBGaXNoZXJpZXMgYW5kIEFxdWF0aWMgU2NpZW5jZXMgbW90b3IgYW5kIGEgd2F0ZXIgZmxvdyBvdXRwdXQgcmF0ZSBvZiAxLDAwMCBML2ggd2FzIHVzZWQgdG8gZ2VuZXJhdGUgbmFub2J1YmJsZXMuIFRoZSBtYWNoaW5lIHByb2R1Y2VkIG5hbm9idWJibGVzICB3aXRoIGFuIGF2ZXJhZ2Ugc2l6ZSBvZiAxNjguOSDCsSA3My44IG5tLCBhbmQgYSBjb25jZW50cmF0aW9uIG9mICAxLjA0IMOXIDEwIDkgwrEgMi42IMOXIDEwIDggcGFydGljbGVzL21MIChBcXVhUHJvIFNvbHV0aW9ucywgMjAxOSkuIFRvIGNyZWF0ZSBveHlnZW4gbmFub2J1YmJsZXMsIHdlIHVzZWQgYW4gb3h5Z2VuIGNvbmNlbnRyYXRvciAobW9kZWwgWXV3ZWxsIDdGLTEwLCBZdXdlbGwsIERhbnlhbmcsIENoaW5hOyAxMCBML21pbiBtYXggIGZsb3dyYXRlLCDiiaUgOTUuNSUgb3h5Z2VuIGNvbmNlbnRyYXRpb24sIGZsb3cgcmF0ZSBhdCA0IEwvbWluKSAgcHJvdmlkZWQgb3h5Z2VuIGZvciBuYW5vYnViYmxlIGdlbmVyYXRvci4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo4Ni4yMjE0MDUwMjkyOTY4OCwibGVmdCI6NTQuMDQ3NTUwMjAxNDE2MDE2LCJ3aWR0aCI6MzguMjAxMjQ4MTY4OTQ1MzEsImhlaWdodCI6MS42OTk2MzEzMzMzNTExMzUzfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6ODcuOTg1Mjk4MTU2NzM4MjgsImxlZnQiOjUxLjE5MDQwNjc5OTMxNjQwNiwid2lkdGgiOjQxLjA2MDEyMzQ0MzYwMzUxNiwiaGVpZ2h0IjoxLjY5OTYzMTMzMzM1MTEzNTN9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo5NC45ODc5Mzc5MjcyNDYxLCJsZWZ0Ijo4LjA5NTIzNTgyNDU4NDk2MSwid2lkdGgiOjg0LjI4NTcyMDgyNTE5NTMxLCJoZWlnaHQiOjEuNjYxNzUwNjc0MjQ3NzQxN319LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjMuNzI4NDYxMjY1NTYzOTY1LCJsZWZ0Ijo4LjEwNDc2MjA3NzMzMTU0Mywid2lkdGgiOjE2LjI1ODc2OTk4OTAxMzY3MiwiaGVpZ2h0IjoxLjI2NjgxMDUzNjM4NDU4MjV9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo0LjcxOTI3MjEzNjY4ODIzMiwibGVmdCI6NzAuNDM0NzIyOTAwMzkwNjIsIndpZHRoIjoxNC4wMzA2OTAxOTMxNzYyNywiaGVpZ2h0IjoxLjMwMDc2OTMyOTA3MTA0NX19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjEwLjM3NDIwNDYzNTYyMDExNywibGVmdCI6OC4wOTUyMzU4MjQ1ODQ5NjEsIndpZHRoIjo0MS4wNzg3NTgyMzk3NDYwOTQsImhlaWdodCI6Ni45OTEyOTg2NzU1MzcxMDl9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjoxNS41OTYyMjc2NDU4NzQwMjMsImxlZnQiOjE0LjE2NzcxNDExODk1NzUyLCJ3aWR0aCI6Ny42NDA0NTYxOTk2NDU5OTYsImhlaWdodCI6MS43NjkyNzUxODg0NDYwNDV9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjoxNS41OTYyMjc2NDU4NzQwMjMsImxlZnQiOjIxLjc3NzMwOTQxNzcyNDYxLCJ3aWR0aCI6MjcuMzc3NTMyOTU4OTg0Mzc1LCJoZWlnaHQiOjEuNzY5Mjc1MTg4NDQ2MDQ1fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MTcuNDI5NzU4MDcxODk5NDE0LCJsZWZ0Ijo4LjA5NTIzNTgyNDU4NDk2MSwid2lkdGgiOjQxLjA2MDMwNjU0OTA3MjI2NiwiaGVpZ2h0IjoxLjY5OTYzMzEyMTQ5MDQ3ODV9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjoxOS4xOTM2NDU0NzcyOTQ5MjIsImxlZnQiOjguMDk1MjM1ODI0NTg0OTYxLCJ3aWR0aCI6NDEuMDY5ODY5OTk1MTE3MTksImhlaWdodCI6MS42OTk2MzMxMjE0OTA0Nzg1fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MjAuOTU3NTM0NzkwMDM5MDYyLCJsZWZ0Ijo4LjA5NTIzNTgyNDU4NDk2MSwid2lkdGgiOjQxLjA1ODc3Njg1NTQ2ODc1LCJoZWlnaHQiOjEuNjk5NjMzMTIxNDkwNDc4NX19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjIyLjcyMTQyNDEwMjc4MzIwMywibGVmdCI6OC4wOTUyMzU4MjQ1ODQ5NjEsIndpZHRoIjo0MC43MTQyOTA2MTg4OTY0ODQsImhlaWdodCI6MS42OTk2MzMxMjE0OTA0Nzg1fX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNDMwIn19XQ%3D%3D. The ROS-mediated oxidation disrupts bacterial membranes, inducing lysis without harming host tissues at optimal dosages. Similarly, oxygen nanobubbles have demonstrated antiviral potential; while direct evidence on WSSV is emerging, analogous mechanisms-ROS-induced viral envelope degradation-mirror successes in wastewater disinfection where nanobubbles eliminate enveloped viruses (Svetovoy, 2021)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlYuIEIuIiwiZmFtaWx5IjoiU3ZldG92b3kifX1dLCJpZHMiOnsiaXNzbiI6WyIxMzU5LTAyOTQiLCIxODc5LTAzOTkiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMTYvai5jb2Npcy4yMDIxLjEwMTQyMyIsIm1hZyI6IjMxMjY0ODQ4NDgiLCJvcGVuYWxleCI6IlczMTI2NDg0ODQ4In0sInRpdGxlIjoiU3BvbnRhbmVvdXMgY2hlbWljYWwgcmVhY3Rpb25zIGJldHdlZW4gaHlkcm9nZW4gYW5kIG94eWdlbiBpbiBuYW5vYnViYmxlcyIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2ouY29jaXMuMjAyMS4xMDE0MjMiLCJyZXNvdXJjZVVybCI6Imh0dHA6Ly9hcnhpdi5vcmcvcGRmLzIxMDIuMDMxMjYiLCJpc3N1ZWREYXRlIjoiMjAyMS0wMi0wNSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkN1cnJlbnQgT3BpbmlvbiBpbiBDb2xsb2lkICYgSW50ZXJmYWNlIFNjaWVuY2UiLCJ2b2x1bWUiOjUyfSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjoxMDE0MjMsImxhc3RQYWdlIjoxMDE0MjN9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRWxzZXZpZXIgQlYifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjYxLCJjaXRlZEJ5Q291bnQiOjJ9fSwicXVvdGUiOiJUaGlzIGF0dGVudGlvbiBpcyBkcml2ZW4gYnkgYSBmdW5kYS1tZW50YWwgcHJvYmxlbTogTkJzIGxpdmUgbXVjaCB0b28gbG9uZyBpbiBjb21wYXJpLXNvbiB3aXRoIHRoZSBwcmVkaWN0aW9uIGdpdmVuIGJ5IHRoZSBkaWZmdXNpb24gZGlzc29sdS10aW9uIHRoZW9yeSBbNCwgNV0uIE9uIHRoZSBvdGhlciBoYW5kLCBleGNpdGluZyBhcHBsaWNhLXRpb25zIG9mIE5CcyBpbiBlbnZpcm9ubWVudCBbNiwgN10sIGFncmljdWx0dXJlIFs4XSwgZmxvdGEtdGlvbiBbOSwgMTBdLCBoZWFsdGhjYXJlIFsxMSwgMTJdLCBhbmQgY2hlbWlzdHJ5IFsxM10gaGF2ZSBlbWVyZ2VkIHRvIG5hbWUgb25seSBhIGZldy4gSW4gY29udHJhc3Qgd2l0aCBzdXJmYWNlIE5Ccywgd2hpY2ggZXhpc3Qgb24gdGhlIHNvbGlkLWxpcXVpZCBpbnRlcmZhY2UsIGJ1bGsgTkJzIGZpbGwgdGhlIHZvbHVtZSBvZiBsaXF1aWQuIFRoZXNlIE5CcyBhcmUgZ2VuZXJhdGVkIGJ5IG1vc3RseSBtZWNoYW5pY2FsIGluamVjdGlvbiBvZiBkaWYtZmVyZW50IGdhc2VzIFs5XSBvciBieSB1bHRyYXNvbmljIGNhdml0YXRpb24gWzEwXSwgYnV0IGh5LWRyb2dlbiBhbmQgb3h5Z2VuIE5CcyB3aGVyZSBhbHNvIGdlbmVyYXRlZCBieSBEQyBlbGVjdHJvbHlzaXMgWzE0LCAxNV0uIExhdGVyIHRoZXNlIG1ldGhvZHMgd2lsbCBiZSBjYWxsZWQgdGhlIHJlZ3VsYXIgbWV0aG9kcy4gVGhlIHNpemUgZGlzdHJpYnV0aW9uIG9mIHRoZSBOQnMgd2FzIGNoYXJhY3Rlcml6ZWQgYnkgdGhlIGR5bmFtaWMgbGlnaHQgc2NhdHRlcmluZyBhbmQgaW5kaXZpZHVhbCBidWJibGVzIHdlcmUgb2JzZXJ2ZWQgYnkgZWxlY3Ryb24gbWljcm9zY29weSBmcm9tIGZyZWV6ZSBmcmFjdHVyZSByZXBsaWNhcyBbMTZdLCBieSBwaGFzZSBtaWNyb3Njb3B5IGFuZCBwb2xhcmltZXRyaWMgc2NhdHRlcm9tZXRyeSBbMTddLiBVbmV4cGVjdGVkbHksIE5CcyBjb250YWluaW5nIG94eWdlbiBhcmUgYWJsZSB0byBwcm8tZHVjZSByZWFjdGl2ZSBveHlnZW4gc3BlY2llcyB3aGljaCBjYW4gb3hpZGl6ZSBwb2xsdS10YW50cyBhbmQgcGF0aG9nZW5zIGluIHdhdGVyIFsxOOKAkzIwXSwgd2hpbGUgc2VwYXJhdGUgb3h5LWdlbiBtb2xlY3VsZXMgaGF2ZSBubyBlZmZlY3Qgb24gb3JnYW5pY3MuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6MzYuOTUwNjQ1NDQ2Nzc3MzQ0LCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA1MjI2MTM1MjUzOTA2LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjM4LjM5NzIzNTg3MDM2MTMzLCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA0ODk5OTc4NjM3Njk1LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjM5Ljg0MzgyNjI5Mzk0NTMxLCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA1MjI2MTM1MjUzOTA2LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjQxLjI5MDQxNjcxNzUyOTMsImxlZnQiOjguODIzNTI1NDI4NzcxOTczLCJ3aWR0aCI6NDAuMDUzODgyNTk4ODc2OTUsImhlaWdodCI6MS4yNTc5MDU5NjAwODMwMDc4fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NDIuNzM3MDEwOTU1ODEwNTUsImxlZnQiOjguODIzNTI1NDI4NzcxOTczLCJ3aWR0aCI6NDAuMDUyMjQ2MDkzNzUsImhlaWdodCI6MS4yNTc5MDU5NjAwODMwMDc4fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NDQuMTgzNjAxMzc5Mzk0NTMsImxlZnQiOjguODIzNTI1NDI4NzcxOTczLCJ3aWR0aCI6NDAuMDUyMjQ2MDkzNzUsImhlaWdodCI6MS4yNTc5MDU5NjAwODMwMDc4fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NDUuNjMwMTg3OTg4MjgxMjUsImxlZnQiOjguODIzNTI1NDI4NzcxOTczLCJ3aWR0aCI6MjAuNTMyMzk4MjIzODc2OTUzLCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjQ3LjA3Njc4MjIyNjU2MjUsImxlZnQiOjEwLjQ1MTQ2NjU2MDM2Mzc3LCJ3aWR0aCI6MzguNDE5NDk4NDQzNjAzNTE2LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjQ4LjUyMzM3MjY1MDE0NjQ4NCwibGVmdCI6OC44MjM1MjU0Mjg3NzE5NzMsIndpZHRoIjo0MC4wNTM4ODY0MTM1NzQyMiwiaGVpZ2h0IjoxLjI1NzkwNTk2MDA4MzAwNzh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo0OS45Njk5NjY4ODg0Mjc3MzQsImxlZnQiOjguODIzNTI1NDI4NzcxOTczLCJ3aWR0aCI6NDAuMDQ3Mzc0NzI1MzQxOCwiaGVpZ2h0IjoxLjI1NzkwNTk2MDA4MzAwNzh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo1MS40MTY1NTczMTIwMTE3MiwibGVmdCI6OC44MjM1MjU0Mjg3NzE5NzMsIndpZHRoIjo0MC4wNTIyNDYwOTM3NSwiaGVpZ2h0IjoxLjI1NzkwNTk2MDA4MzAwNzh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo1Mi44NjMxNDM5MjA4OTg0NCwibGVmdCI6OC44MjM1MjU0Mjg3NzE5NzMsIndpZHRoIjo0MC4wNDczNzQ3MjUzNDE4LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjU0LjMwOTczODE1OTE3OTY5LCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA0NDExMzE1OTE3OTY5LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjU1Ljc1NjMzMjM5NzQ2MDk0LCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA0OTAxMTIzMDQ2ODc1LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjU3LjIwMjkxOTAwNjM0NzY1NiwibGVmdCI6OC44MjM1MjU0Mjg3NzE5NzMsIndpZHRoIjo0MC4wNDczNzQ3MjUzNDE4LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjU4LjY0OTUwOTQyOTkzMTY0LCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA0OTAwMzYwMTA3NDIyLCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjYwLjA5NjEwMzY2ODIxMjg5LCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA0NTc0NTg0OTYwOTM3NSwiaGVpZ2h0IjoxLjI1NzkwNTk2MDA4MzAwNzh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2MS41NDI2OTAyNzcwOTk2MSwibGVmdCI6OC44MjM1MjU0Mjg3NzE5NzMsIndpZHRoIjoyNS43MTIzMDMxNjE2MjEwOTQsImhlaWdodCI6MS4yNTc5MDU5NjAwODMwMDc4fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NjIuOTg5NDA2NTg1NjkzMzYsImxlZnQiOjEwLjQ1MTQ2NjU2MDM2Mzc3LCJ3aWR0aCI6MzguNDIxMTEyMDYwNTQ2ODc1LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjY0LjQzNTk5NzAwOTI3NzM0LCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA0NTc0NTg0OTYwOTM3NSwiaGVpZ2h0IjoxLjI1NzkwNTk2MDA4MzAwNzh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2NS44ODI1OTEyNDc1NTg2LCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA0ODk5NTk3MTY3OTY5LCJoZWlnaHQiOjEuMjU3OTAyMTQ1Mzg1NzQyMn19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjY3LjMyOTE5MzExNTIzNDM4LCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjMwLjY2OTE3OTkxNjM4MTgzNiwiaGVpZ2h0IjoxLjI1NzkwMjE0NTM4NTc0MjJ9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIxMDE0MjMifX1d. Parasitic control benefits from biophysical effects, including altered interfacial tension that detaches protozoans like Ichthyophthirius multifiliis from fish gills. Field applications in shrimp ponds report up to 90% reduction in Vibrio counts post-nanobubble treatment, mitigating acute hepatopancreatic necrosis disease outbreaks (Nghĩa et al., 2022)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5ndXnhu4VuIEjhu691IiwiZmFtaWx5IjoiTmdoxKlhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5ndXllbiBUaGkiLCJmYW1pbHkiOiJOZ3V5ZW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUGhhbiBUcuG7jW5nIiwiZmFtaWx5IjoiQsOsbmgifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTGUgVGhpIiwiZmFtaWx5IjoiTWF5In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRvbmcgVHJhbiIsImZhbWlseSI6Ikh1eSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQaOG6oW0gVGjDoWkiLCJmYW1pbHkiOiJHaWFuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTb3BoaWUiLCJmYW1pbHkiOiJTdOKAkEhpbGFpcmUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUGhhbiBUaGkiLCJmYW1pbHkiOiJWYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMjM0LTE3NDkiLCIyMjM0LTE3NTciXSwiaXNibiI6W10sImRvaSI6IjEwLjQ3ODUzL2Zhcy4yMDIyLmUzOSIsIm9wZW5hbGV4IjoiVzQyOTQ1OTc1NTQifSwidGl0bGUiOiJFZmZlY3Qgb2YgbmFub2J1YmJsZXMgKG94eWdlbiwgb3pvbmUpIG9uIHRoZSBQYWNpZmljIHdoaXRlIHNocmltcCAoUGVuYWV1cyB2YW5uYW1laSksIFZpYnJpbyBwYXJhaGFlbW9seXRpY3VzIGFuZCB3YXRlciBxdWFsaXR5IHVuZGVyIGxhYiBjb25kaXRpb25zIiwiYWJzdHJhY3QiOiJUaGlzIHN0dWR5IGFzc2Vzc2VkIHRoZSBlZmZlY3RzIG9mIG94eWdlbiBhbmQgb3pvbmUgbmFub2J1YmJsZXMgb24gZ2lsbCBtb3JwaG9sb2d5LCB3ZWlnaHQgZ2FpbiwgYW5kIG1vcnRhbGl0eSBvZiBQYWNpZmljIHdoaXRlIHNocmltcCAoUGVuYWV1cyB2YW5uYW1laSksIGFzIHdlbGwgYXMgdGhlIGxldmVsIG9mIFZpYnJpbyBwYXJhaGFlbW9seXRpY3VzIGFuZCB3YXRlciBxdWFsaXR5IG9mIHNocmltcCBjdWx0dXJlIHRhbmtzIHVuZGVyIGxhYiBjb25kaXRpb25zLiBUd28gZXhwZXJpbWVudHMgd2VyZSBjYXJyaWVkIG91dCB3aXRoIG94eWdlbiBtYWNyb2J1YmJsZSwgb3pvbmUgbWFjcm9idWJibGUsIG94eWdlbiBuYW5vYnViYmxlLCBvem9uZSBuYW5vYnViYmxlLCBhbmQgY29udHJvbCB0cmVhdG1lbnRzIChhaXItc3RvbmUgbWFjcm9idWJibGUpLiBFeHBlcmltZW50cyB3ZXJlIGRvbmUgaW4gdHJpcGxpY2F0ZSBpbiAxMDAgTCB0YW5rcyB3aXRoIDE14oCwIHNhbGluZSB3YXRlciwgYW5kIDIwIHNocmltcCBwZXIgdGFuay4gVGFua3MgaW4gRXhwZXJpbWVudCAxIHdlcmUgbm90IGlub2N1bGF0ZWQgd2l0aCBiYWN0ZXJpYTsgdGFua3MgaW4gRXhwZXJpbWVudCAyIHdlcmUgaW5vY3VsYXRlZCB3aXRoIFYuIHBhcmFoYWVtb2x5dGljdXMgYXQgYSBjb25jZW50cmF0aW9uIG9mIDEwNiBDRlUvbUwuIFRoZSByZXN1bHRzIHJldmVhbGVkIHRoYXQgc2hvcnQgdHJlYXRtZW50cyB3aXRoIG96b25lIG5hbm9idWJibGVzIGhhZCBtaW5pbWFsIGltcGFjdCBvbiBzaHJpbXAgZ2lsbHMsIG1vcnRhbGl0eSwgYW5kIGdyb3d0aCByYXRlcywgcmVkdWNlZCBWLiBwYXJhaGFlbW9seXRpY3VzIGNvbmNlbnRyYXRpb24gaW4gd2F0ZXIgY29tcGFyZWQgdG8gdGhlIG90aGVyIGdyb3VwcywgYW5kIGltcHJvdmUgd2F0ZXIgcXVhbGl0eS4gVGhlc2UgbGFib3JhdG9yeSByZXN1bHRzIGluZGljYXRlIHRoYXQgb3pvbmUgbmFub2J1YmJsZSB0cmVhdG1lbnQgbWF5IGJlIHVzZWZ1bCBmb3IgY29udHJvbGxpbmcgVi4gcGFyYWhhZW1vbHl0aWN1cy4gTW9yZSB3b3JrIGlzIG5lZWRlZCB0byBmaW5kIHRoZSBiZXN0IHByb3RvY29sIHRvIGFwcGx5IHRoZSB0ZWNobm9sb2d5IG9uIGEgY29tbWVyY2lhbCBzY2FsZS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuNDc4NTMvZmFzLjIwMjIuZTM5IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5lLWZhcy5vcmcvZG93bmxvYWQvZG93bmxvYWRfcGRmP3BpZD1mYXMtMjUtOC00MjkiLCJpc3N1ZWREYXRlIjoiMjAyMi0wOC0wMSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZpc2hlcmllcyBhbmQgYXF1YXRpYyBzY2llbmNlcyIsImlzc3VlIjo4LCJ2b2x1bWUiOjI1LCJpbXBhY3RGYWN0b3IiOiIxLjM0In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6NDI5LCJsYXN0UGFnZSI6NDQwfX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlNwcmluZ2VyIFNjaWVuY2UrQnVzaW5lc3MgTWVkaWEifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjMwLCJjaXRlZEJ5Q291bnQiOjEzfX0sInF1b3RlIjoiVGhlIGluZHVzdHJ5IHRoZXJlZm9yZSBuZWVkcyB0byBmaW5kIGFsdGVybmF0aXZlIG1lYXN1cmVzICB0byBwcmV2ZW50IGFuZCBjb250cm9sIHRoaXMgcGF0aG9nZW4uIE5hbm9idWJibGUgdGVjaG5vbG9neSBwcm9kdWNlcyBzbWFsbCBnYXMgYnViYmxlcyB3aXRoIGEgZGlhbWV0ZXIgb2YgbGVzcyB0aGFuIDIwMCBubSAoQWdyYXdhbCBldCBhbC4sIDIwMTE7IENoYXBsaW4sIDIwMjEpIHdoaWNoIGNhbiBleGlzdCBpbiB0aGUgd2F0ZXIgY29sdW1uIGZvciB3ZWVrcyAoQXpldmVkbyAgZXQgYWwuLCAyMDE2OyBQYXJtYXIgJiBNYWp1bWRlciwgMjAxMykuIFRoaXMgcGhlbm9tZW5vbiBpcyAgYXNzb2NpYXRlZCB3aXRoIHRoZSBwaHlzaWNhbCBjaGFyYWN0ZXJpc3RpYyBvZiB0aGVzZSBzbWFsbCBidWItYmxlcyAoV2VpanMgJiBMb2hzZSwgMjAxMykgYW5kIGhpZ2ggemV0YSBwb3RlbnRpYWwgdmFsdWUgd2hpY2ggIGtlZXBzIHRoZW0gZnJvbSBhZ2dyZWdhdGluZyBpbnRvIGxhcmdlciBidWJibGVzIChLaXJieSwgMjAxMDsgIFNqb2dyZWVuIGV0IGFsLiwgMjAxOCkuIE5hbm9idWJibGUgdGVjaG5vbG9neSBoYXMgYSBsb3Qgb2YgcHJvbS1pc2luZyBlZmZlY3RpdmUgYXBwbGljYXRpb24gaW4gYXF1YWN1bHR1cmUgYmVjYXVzZSBvZiBpdHMgYWJpbGl0eSAgdG8gaW5jcmVhc2UgYW5kIG1haW50YWluIG94eWdlbiBjb250ZW50IGluIHdhdGVyIGFuZCBjb250cm9sICBiYWN0ZXJpYSwgYSBwb3NzaWJsZSBtZWFzdXJlIHRvIHJlZHVjZSB0aGUgdXNlIG9mIGFudGliaW90aWNzLiBPeHlnZW4gbmFub2J1YmJsZXMgb3B0aW1pemUgdGhlIGRpc3NvbHZlZCBveHlnZW4gaW4gYXF1YXRpYyBzeXN0ZW1zIChHYWxhbmcgZXQgYWwuLCAyMDE5KS4gSGlnaCBsZXZlbHMgb2Ygb3h5Z2VuIG1heSBzcGVlZCB1cCBtZXRhYm9saXNtLCBwcm9tb3RlIGhpZ2hlciBmb29kIGludGFrZSwgYW5kIGluY3JlYXNlICBncm93dGggb2YgZmFybWVkIHNocmltcCAoRWJpbmEgZXQgYWwuLCAyMDEzKS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNS4wNjg2NDU0NzcyOTQ5MiwibGVmdCI6OC4wOTUyMzU4MjQ1ODQ5NjEsIndpZHRoIjo0MS4wNzIyMTIyMTkyMzgyOCwiaGVpZ2h0IjoxLjY5OTYzNTE0ODA0ODQwMDl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNi44MzI1MzA5NzUzNDE4LCJsZWZ0Ijo4LjA5NTIzNTgyNDU4NDk2MSwid2lkdGgiOjIzLjYwNDE4NTEwNDM3MDExNywiaGVpZ2h0IjoxLjY5OTYzNTE0ODA0ODQwMDl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC41OTY0MjAyODgwODU5NCwibGVmdCI6MTAuOTUyMzY0OTIxNTY5ODI0LCJ3aWR0aCI6MzguMjE1ODc3NTMyOTU4OTg0LCJoZWlnaHQiOjEuNjk5NjM1MTQ4MDQ4NDAwOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQwLjM2MDMwOTYwMDgzMDA4LCJsZWZ0Ijo4LjA5NTIzNTgyNDU4NDk2MSwid2lkdGgiOjQxLjA3Njc1OTMzODM3ODkwNiwiaGVpZ2h0IjoxNy41NzQ2MzA3MzczMDQ2ODh9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1Ny45OTkxOTUwOTg4NzY5NSwibGVmdCI6MTAuOTUyMzY0OTIxNTY5ODI0LCJ3aWR0aCI6MzguMjAxNDQ2NTMzMjAzMTI1LCJoZWlnaHQiOjEuNjk5NjM1MTQ4MDQ4NDAwOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjU5Ljc2MzA4MDU5NjkyMzgzLCJsZWZ0Ijo4LjA5NTIzNTgyNDU4NDk2MSwid2lkdGgiOjQxLjA2MzMyNzc4OTMwNjY0LCJoZWlnaHQiOjEuNjk5NjM1MTQ4MDQ4NDAwOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjYxLjUyNjk3MzcyNDM2NTIzNCwibGVmdCI6OC4wOTUyMzU4MjQ1ODQ5NjEsIndpZHRoIjo0MS4wNjY4NDQ5NDAxODU1NSwiaGVpZ2h0IjoxLjY5OTYzNTE0ODA0ODQwMDl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2My4yOTA4NjMwMzcxMDkzNzUsImxlZnQiOjguMDk1MjM1ODI0NTg0OTYxLCJ3aWR0aCI6NDAuNzE0MjkwNjE4ODk2NDg0LCJoZWlnaHQiOjEuNjk5NjMxMzMzMzUxMTM1M319XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjQzMCJ9fV0%3D. These outcomes stem from nanobubbles' prolonged stability (days to weeks), enabled by high zeta potentials repelling coalescence, ensuring sustained ROS delivery throughout water columns (Tekile et al., 2016)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFuZGluZXQgS2ViZWRlIiwiZmFtaWx5IjoiVGVraWxlIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikls4oCQSG8iLCJmYW1pbHkiOiJLaW0ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSmFpeWVvcCIsImZhbWlseSI6IkxlZSJ9fV0sImlkcyI6eyJpc3NuIjpbIjEyMjYtMTAyNSIsIjIwMDUtOTY4WCJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuNDQ5MS9lZXIuMjAxNi4wMjgiLCJtYWciOiIyNTIzNjY5NjU3Iiwib3BlbmFsZXgiOiJXMjUyMzY2OTY1NyJ9LCJ0aXRsZSI6IkV4dGVudCBhbmQgcGVyc2lzdGVuY2Ugb2YgZGlzc29sdmVkIG94eWdlbiBlbmhhbmNlbWVudCB1c2luZyBuYW5vYnViYmxlcyIsImFic3RyYWN0IjoiSW4gdGhpcyBzdHVkeSwgY2hhbmdlIGluIHdhdGVyLWRpc3NvbHZlZCBveHlnZW4gKERPKSB3YXMgYW5hbHl6ZWQgdW5kZXIgdmFyaW91cyBzeW50aGV0aWMgd2F0ZXIgcXVhbGl0aWVzIGFuZCBuYW5vYnViYmxlcyAoTkJzKSBhcHBsaWNhdGlvbiBjb25kaXRpb25zLCBzdWNoIGFzIGdhcyB0eXBlLCBpbml0aWFsIERPIGFzIHdlbGwgYXMgd2F0ZXIgZGlzc29sdmVkLCBzdXNwZW5kZWQgYW5kIG9yZ2FuaWMgbWF0dGVycyBjb250ZW50cy4gV2hlbiBveHlnZW4sIHJhdGhlciB0aGFuIGFpciwgd2FzIGludHJvZHVjZWQgaW50byBuaXRyb2dlbi1kZXNvcmJlZCB1bHRyYS1wdXJlIHdhdGVyLCB0aGUgc3RhZ25hdGlvbiB0aW1lIHdhcyBzaWduaWZpY2FudGx5IGluY3JlYXNlZC4gSXQgdG9vayB0ZW4gZGF5cyBmb3IgRE8gY29uY2VudHJhdGlvbiB0byBkcm9wIGJhY2sgdG8gc2F0dXJhdGlvbi4gVGhlIGhpZ2hlciB0aGUgaW5pdGlhbCBETyBjb25jZW50cmF0aW9uLCB0aGUgbG9uZ2VyIHBhcnRpY2xlcyB3ZXJlIG9ic2VydmVkIGFib3ZlIHNhdHVyYXRpb24gZHVlIHRvIHBhcnRpY2xlIHN0YWJpbGl0eSBpbXByb3ZlbWVudC4gVGhlIG94eWdlbiBtYXNzIHRyYW5zZmVyIHJhdGUgb2YgMC4wNDgyIG1nL0wvbWluIHdhcyBmb3VuZCB0byByZWFjaCBhIG1heGltdW0gYXQgYW4gZWxlY3Ryb2x5dGljIGNvbmNlbnRyYXRpb24gb2YgMC43NSBnL0wsIGJleW9uZCB3aGljaCB0aGUgdHJhbnNmZXIgcmF0ZSBkZWNyZWFzZWQgZHVlIHRvIGFkc29ycHRpb24gb2YgbmVnYXRpdmUgaW9ucyBvZiB0aGUgZWxlY3Ryb2x5dGUgYXQgdGhlIGludGVyZmFjZS4gSGlnaCBsZXZlbHMgb2YgdHVyYmlkaXR5IGNhdXNlZCBieSBzdXNwZW5kZWQgc29saWRzIGhhdmUgYmVjb21lIGEgYmFycmllciB0byBkaXNzb2x1dGlvbiBvZiBOQnMgb3h5Z2VuIGludG8gdGhlIHdhdGVyIHNvbHV0aW9uLCBhbmQgdGh1cyBhZmZlY3RlZCB0aGUgdHJhbnNmZXIgcGVyZm9ybWFuY2UuIE9uIHRoZSBvdGhlciBoYW5kLCBieSBhcHBseWluZyBOQnMgZm9yIGp1c3QgYW4gaG91ciwgdXAgdG8gNy4yJSBkZWdyYWRhdGlvbiBvZiBnbHVjb3NlIGFzIHJlcHJlc2VudGF0aXZlIG9yZ2FuaWMgbWF0dGVyIHdhcyBhY2hpZXZlZC4gVGh1cywgTkJzIHRlY2hub2xvZ3kgd291bGQgbWFpbnRhaW4gYSBoaWdoIERPIGV4dGVudCBmb3IgYW4gZXh0ZW5kZWQgZHVyYXRpb24sIGFuZCB0aHVzIGNhbiBpbXByb3ZlIHdhdGVyIHF1YWxpdHkgcHJvdmlkZWQgdGhhdCB3YXRlciBjaGVtaXN0cnkgaXMgY2xvc2VseSBtb25pdG9yZWQgZHVyaW5nIGl0cyBhcHBsaWNhdGlvbi4gS2V5d29yZHM6IEFwcGxpY2F0aW9uIGNvbmRpdGlvbnMsIERpc3NvbHZlZCBveHlnZW4sIE5hbm9idWJibGVzLCBXYXRlciBxdWFsaXR5IiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjQ0OTEvZWVyLjIwMTYuMDI4IiwicmVzb3VyY2VVcmwiOiJodHRwOi8vZWVlci5vcmcvdXBsb2FkL2Vlci0yMS00LTQyNy5wZGYiLCJpc3N1ZWREYXRlIjoiMjAxNi0wOS0xMiIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkVudmlyb25tZW50YWwgRW5naW5lZXJpbmcgUmVzZWFyY2giLCJpc3N1ZSI6NCwidm9sdW1lIjoyMSwiaW1wYWN0RmFjdG9yIjoiNC4wNSJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjQyNywibGFzdFBhZ2UiOjQzNX19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJLb3JlYW4gU29jaWV0eSBvZiBFbnZpcm9ubWVudGFsIEVuZ2luZWVyaW5nIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjoyNCwiY2l0ZWRCeUNvdW50IjozM319LCJxdW90ZSI6IlRoaXMgdmVyaWZpZXMgdGhlIHBvc3NpYmlsaXR5IG9mIHRoZSBleGlzdGVuY2UgIG9mIE5Ccywgc2luY2UgYXQgYSBoaWdoZXIgRE8gY29uY2VudHJhdGlvbiwgdGhlIG94eWdlbiBkaWZmdXNpb24gIHdvdWxkIHRha2UgbW9yZSB0aW1lLiBUaGUgaGlnaCBnYXMgY29uY2VudHJhdGlvbiBpbiB0aGUgbGlxdWlkICBleHBsYWlucyB0aGUgZXh0ZW5zaW9uIG9mIE5CIHN0YWJpbGl0eS4gIFRoZSBtYWduaXR1ZGVzIG9mIHpldGEgcG90ZW50aWFsIG9mIG94eWdlbiBuYW5vLWJ1YmJsZWQgd2F0ZXIgIHdlcmUgcG9zc2libHkgaGlnaGVyIHRoYW4gdGhvc2Ugb2YgYWlyIG5hbm8tYnViYmxlZCB3YXRlci4gQXQgIGEgaGlnaCBtYWduaXR1ZGUgemV0YSBwb3RlbnRpYWwgdmFsdWUsIHRoZSBlbGVjdHJpY2FsbHkgY2hhcmdlZCAgcGFydGljbGVzIHRlbmQgdG8gcmVwZWwgZWFjaCBvdGhlciwgYXZvaWRpbmcgYWdncmVnYXRpb24uIEZvciBidWJibGUgIGRpc3BlcnNpb24sIHRoZSBoaWdoIHpldGEgcG90ZW50aWFsIGNvdWxkIGNyZWF0ZSByZXB1bHNpb24gZm9yY2VzICB0aGF0IHdvdWxkIGF2b2lkIHRoZSBjb2FsZXNjZW5jZSBvZiBidWJibGVzLCB0aHVzIGNvbnRyaWJ1dGluZyAgdG8gdGhlIHN0YWJpbGl6YXRpb24gb2YgdGhlIGJ1YmJsZXMuIFRoZXJlZm9yZSwgdGhlIG94eWdlbiBOQnMgIGNvdWxkIGJlIHN0YWJpbGl6ZWQgaW4gd2F0ZXIgYnkgdGhlIGVsZWN0cmljYWxseSBjaGFyZ2VkIHN1cmZhY2VzLCAgd2hpbGUgdGhlIG5hbm8tYnViYmxlZCBhaXIgd291bGQgYmUgYmVsb3cgdGhlIGxpbWl0IG9mIHRoZSBzdGFiaWxpemEtdGlvbiBieSByZXB1bHNpb24gZm9yY2VzLiBGdXJ0aGVybW9yZSwgdGhlIGJpZyBkaWZmZXJlbmNlIGJldHdlZW4gdGhlIHBlcmlvZHMgaW4gd2hpY2ggIGJ1YmJsZXMgY291bGQgYmUgb2JzZXJ2ZWQgYXMgc3RhYmxlIGluIHRoZSBveHlnZW4gbmFuby1idWJibGVkICB3YXRlciAoMTAtMTUgZCkgYW5kIGluIHRoYXQgb2YgYWlyIG5hbm8tYnViYmxlZCAoMy01IGQpIGlzIGxpa2VseSBGaWcuIDYuIEdyYXBocyBzaG93aW5nIHN0YWduYXRpb24gdGltZSBmb3IgZGlmZmVyZW50IGluaXRpYWwgZGlzc29sdmVkIG94eWdlbiAoRE8pLiByZWxhdGVkIHRvIHRoZSBsb3dlciBkaXNzb2x1dGlvbiBvZiB0aGUgYWlyIGluIHdhdGVyIGluIHJlbGF0aW9uIHRvIHRoZSBkaXNzb2x1dGlvbiBvZiBveHlnZW4uICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6NjYuMTc4ODAyNDkwMjM0MzgsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6NDAuNzE0ODg5NTI2MzY3MTksImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo2Ny41NzY3NzQ1OTcxNjc5NywibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42ODgyNzA1Njg4NDc2NTYsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo2OC45NjcyMzE3NTA0ODgyOCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NTcxODA3ODYxMzI4MSwiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjcwLjM1NzY4ODkwMzgwODYsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6MjUuMzQxMjU1MTg3OTg4MjgsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3MS43NTU2NTMzODEzNDc2NiwibGVmdCI6OS41NDk2NDE2MDkxOTE4OTUsIndpZHRoIjozOS4wMjQ3ODQwODgxMzQ3NjYsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3My4xNDYxMDI5MDUyNzM0NCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NTQwMjk4NDYxOTE0MDYsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3NC41MzY1NjAwNTg1OTM3NSwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NTM0NjUyNzA5OTYwOTQsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3NS45MzQ1MzIxNjU1MjczNCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NzMyMzY4NDY5MjM4MywiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjc3LjMyNDk4OTMxODg0NzY2LCJsZWZ0Ijo3Ljg2NTU0NTI3MjgyNzE0OCwid2lkdGgiOjQwLjY0OTM1MzAyNzM0Mzc1LCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6NzguNzE1NDQ2NDcyMTY3OTcsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6NDAuNjU0NjI0OTM4OTY0ODQ0LCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODAuMTEzNDEwOTQ5NzA3MDMsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6NDAuNjUxMDUwNTY3NjI2OTUsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo4MS41MDM4NjA0NzM2MzI4MSwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NTQzNDI2NTEzNjcxOSwiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjgyLjg5NDMxNzYyNjk1MzEyLCJsZWZ0Ijo3Ljg2NTU0NTI3MjgyNzE0OCwid2lkdGgiOjQwLjMyMTgyMzEyMDExNzE5LCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODQuMjkyMjg5NzMzODg2NzIsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6MTUuODcwMjk1NTI0NTk3MTY4LCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODUuNjgyNzQ2ODg3MjA3MDMsImxlZnQiOjkuNTQ5NjQxNjA5MTkxODk1LCJ3aWR0aCI6MzguOTg5NzA0MTMyMDgwMDgsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo4Ny4wNzMyMDQwNDA1MjczNCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC43MTg0OTQ0MTUyODMyLCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODguNDcxMTY4NTE4MDY2NCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42ODc0MTk4OTEzNTc0MiwiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjM0LjUxMzI3MTMzMTc4NzExLCJsZWZ0Ijo1MS45MzI3Nzc0MDQ3ODUxNTYsIndpZHRoIjo0MC42NDcwNTI3NjQ4OTI1OCwiaGVpZ2h0IjoyLjcxODY2NjU1MzQ5NzMxNDV9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjozOC43MjUxNTg2OTE0MDYyNSwibGVmdCI6NTEuOTMyNzc3NDA0Nzg1MTU2LCJ3aWR0aCI6NDAuNjQzMTM1MDcwODAwNzgsImhlaWdodCI6MS4yNTMyODg1MDc0NjE1NDc5fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6NDAuMTE1NjEyMDMwMDI5MywibGVmdCI6NTEuOTMyNzc3NDA0Nzg1MTU2LCJ3aWR0aCI6NDAuNjY0NTg1MTEzNTI1MzksImhlaWdodCI6MS4yNTMyODg1MDc0NjE1NDc5fX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNDMxIn19XQ%3D%3D.

Beyond direct pathogen elimination, nanobubbles modulate microbial communities to favor beneficial taxa, indirectly bolstering disease resistance. Enhanced DO supersaturation (200-300%) curbs anaerobic pathogens while promoting nitrifying bacteria, as evidenced in Penaeus vannamei cultures where total ammonia nitrogen dropped by 9%, alleviating stress-induced immunosuppression (Susanti et al., 2021)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkxpbHkiLCJmYW1pbHkiOiJTdXNhbnRpIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlN1eXVkIFdhcm5vIiwiZmFtaWx5IjoiVXRvbW8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTm92ZXJpdGEgRGlhbiIsImZhbWlseSI6IlRha2FyaW5hIn19XSwiaWRzIjp7Imlzc24iOlsiMjExNy00NDU4IiwiMjI3My0xNzA5Il0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDUxL2Jpb2NvbmYvMjAyMTMzMDUwMDUiLCJtYWciOiIzMTkzNTYzNzAzIiwib3BlbmFsZXgiOiJXMzE5MzU2MzcwMyJ9LCJ0aXRsZSI6IlN1c3RhaW5hYmlsaXR5IGFuZCBmZWFzaWJpbGl0eSBhc3Nlc3NtZW50cyBvZiBuYW5vYnViYmxlIGFlcmF0aW9uIHRlY2hub2xvZ3kgaW4gZWNvbm9taWMtc29jaW8gZW52aXJvbm1lbnQgb2YgUGVuYWV1cyB2YW5uYW1laSBzaHJpbXAgZmFybWluZyIsImFic3RyYWN0IjoiUGVuYWV1cyB2YW5uYW1laSBzaHJpbXAgZmFybWluZyBpbiBJbmRvbmVzaWEgaXMgZmFjZWQgd2l0aCBzZXZlcmFsIGNoYWxsZW5nZXMgaW4gdGhlIGVudmlyb25tZW50YWwsIGVjb25vbWljLCBhbmQgc29jaWFsIGFzcGVjdHMuIFRoZXJlZm9yZSwgdGhpcyBzdHVkeSBhaW1zIHRvIGFzc2VzcyB0aGUgYmVuZWZpdHMgb2Ygbm92ZWwgbmFub2J1YmJsZSBhZXJhdGlvbiBzeXN0ZW1zIGZvciB0aGUgZWNvc3lzdGVtLCBidXNpbmVzc2VzLCBhbmQgY29tbXVuaXRpZXMuIFRoaXMgaXMgYW4gZXhwZXJpbWVudGFsIHN0dWR5IGNvbmR1Y3RlZCBieSBjb21wYXJpbmcgUC4gdmFubmFtZWkgcG9zdCBsYXJ2YSAxMCByZWFyZWQgYXQgYSBkZW5zaXR5IG9mIDIwMDAgaW5kcy4vTCBpbiBhIHBvbmQgdHJlYXRlZCB3aXRoIG5hbm9idWJibGUgYW5kIHdpdGhvdXQgdHJlYXRtZW50cyAoY29udHJvbCkuIEZ1cnRoZXJtb3JlLCB0aGUgTmV0IFByZXNlbnQgVmFsdWUgKE5QVikgYW5kIHBheWJhY2sgcGVyaW9kIHdlcmUgY2FsY3VsYXRlZCB0byBhc3Nlc3MgdGhlIGVjb25vbWljIGZlYXNpYmlsaXR5IG9mIG5hbm9idWJibGUsIHdoaWxlIGNvbW11bml0eSBpbnRlcnZpZXdzIHdlcmUgdXNlZCBmb3IgdGhlIHNvY2lhbCBzZWN0b3JzLiBCYXNlZCBvbiB0aGUgcmVzdWx0cywgZW52aXJvbm1lbnRhbCB3YXN0ZXMgb2Ygc2hyaW1wIGZhcm1pbmcgaW4gZm9ybSBvZiB0b3RhbCBhbW1vbmlhIG5pdHJvZ2VuIChUQU4pIHdlcmUgcmVkdWNlZCBieSA5JSBmcm9tIDIuNTggbWcvbCAoOTUlQ0k6IDAsOTEsIDQsMjUpIGluIGNvbnRyb2wgY29tcGFyZWQgdG8gMi4zNSBtZy9sICg5NSVDSTogMCw4NiwgMyw4NCkgaW4gdHJlYXRtZW50LiBGdXJ0aGVybW9yZSwgdGhlIHJldmVudWVzIGZyb20gcG9zdCBsYXJ2YSBzYWxlcyBmb3IgZml2ZSB5ZWFycyBhbmQgbmFub2J1YmJsZSBpbnZlc3RtZW50IGNvc3RzIHNob3dlZCB0aGF0IHRoZSBlc3RpbWF0ZWQgTlBWIHdhcyBJRFIgNjQsODI0LDM3NCB3aXRoIGEgcGF5YmFjayBwZXJpb2Qgb2YgMS43IHllYXJzLiBUaGUgaW50ZXJ2aWV3cyBvbiB0cmFkaXRpb25hbCBzaHJpbXAgZmFybWluZyBzaG93ZWQgdGhhdCA2MS4xJSB0byA3Mi4yJSBvZiBjb21tdW5pdHkgbWVtYmVycyBhZ3JlZWQgb24gdGhlIHVzZSBvZiBuYW5vYnViYmxlIGFlcmF0aW9uIHRvIHN1cHBvcnQgbGl2ZWxpaG9vZHMuIFRoZXJlZm9yZSwgdGhlIHVzZSBvZiBuYW5vYnViYmxlcyBpcyBmZWFzaWJsZSB0byBzdXBwb3J0IHN1c3RhaW5hYmxlIFAuIHZhbm5hbWVpIGZhcm1pbmcuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjEwNTEvYmlvY29uZi8yMDIxMzMwNTAwNSIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cuYmlvLWNvbmZlcmVuY2VzLm9yZy9hcnRpY2xlcy9iaW9jb25mL3BkZi8yMDIxLzA1L2Jpb2NvbmZfaWNhdmVzczIwMjFfMDUwMDUucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjEtMDEtMDEiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA5IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJCSU8gV2ViIG9mIENvbmZlcmVuY2VzIiwidm9sdW1lIjozMywiaW1wYWN0RmFjdG9yIjoiMC42MiJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjUwMDUsImxhc3RQYWdlIjo1MDA1fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkVEUCBTY2llbmNlcyJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MTEsImNpdGVkQnlDb3VudCI6NH19LCJxdW90ZSI6IkFic3RyYWN0LiBQZW5hZXVzIHZhbm5hbWVpIHNocmltcCBmYXJtaW5nIGluIEluZG9uZXNpYSBpcyBmYWNlZCB3aXRoIHNldmVyYWwgY2hhbGxlbmdlcyBpbiB0aGUgZW52aXJvbm1lbnRhbCwgZWNvbm9taWMsIGFuZCBzb2NpYWwgYXNwZWN0cy4gIFRoZXJlZm9yZSwgdGhpcyBzdHVkeSBhaW1zIHRvIGFzc2VzcyB0aGUgYmVuZWZpdHMgb2Ygbm92ZWwgbmFub2J1YmJsZSAgYWVyYXRpb24gc3lzdGVtcyBmb3IgdGhlIGVjb3N5c3RlbSwgYnVzaW5lc3NlcywgYW5kIGNvbW11bml0aWVzLiBUaGlzIGlzIGFuICBleHBlcmltZW50YWwgc3R1ZHkgY29uZHVjdGVkIGJ5IGNvbXBhcmluZyBQLiB2YW5uYW1laSBwb3N0IGxhcnZhIDEwICByZWFyZWQgYXQgYSBkZW5zaXR5IG9mIDIwMDAgaW5kcy4vTCBpbiBhIHBvbmQgdHJlYXRlZCB3aXRoIG5hbm9idWJibGUgYW5kICB3aXRob3V0IHRyZWF0bWVudHMgKGNvbnRyb2wpLiBGdXJ0aGVybW9yZSwgdGhlIE5ldCBQcmVzZW50IFZhbHVlIChOUFYpIGFuZCAgcGF5YmFjayBwZXJpb2Qgd2VyZSBjYWxjdWxhdGVkIHRvIGFzc2VzcyB0aGUgZWNvbm9taWMgZmVhc2liaWxpdHkgb2YgIG5hbm9idWJibGUsIHdoaWxlIGNvbW11bml0eSBpbnRlcnZpZXdzIHdlcmUgdXNlZCBmb3IgdGhlIHNvY2lhbCBzZWN0b3JzLiAgQmFzZWQgb24gdGhlIHJlc3VsdHMsIGVudmlyb25tZW50YWwgd2FzdGVzIG9mIHNocmltcCBmYXJtaW5nIGluIGZvcm0gb2YgIHRvdGFsIGFtbW9uaWEgbml0cm9nZW4gKFRBTikgd2VyZSByZWR1Y2VkIGJ5IDklIGZyb20gMi41OCBtZy9sICg5NSVDSTogIDAsOTEsIDQsMjUpIGluIGNvbnRyb2wgY29tcGFyZWQgdG8gMi4zNSBtZy9sICg5NSVDSTogMCw4NiwgMyw4NCkgaW4gIHRyZWF0bWVudC4gRnVydGhlcm1vcmUsIHRoZSByZXZlbnVlcyBmcm9tIHBvc3QgbGFydmEgc2FsZXMgZm9yIGZpdmUgeWVhcnMgYW5kICBuYW5vYnViYmxlIGludmVzdG1lbnQgY29zdHMgc2hvd2VkIHRoYXQgdGhlIGVzdGltYXRlZCBOUFYgd2FzIElEUiAgNjQsODI0LDM3NCB3aXRoIGEgcGF5YmFjayBwZXJpb2Qgb2YgMS43IHllYXJzLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjQ1LjQ4MzI4NzgxMTI3OTMsImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2LjgwMDU2NzYyNjk1MzEyNSwiaGVpZ2h0IjoxLjc0OTIzODYxMDI2NzYzOTJ9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo0Ny4xNjcxNzE0NzgyNzE0ODQsImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2Ljc3NTAzOTY3Mjg1MTU2LCJoZWlnaHQiOjEuNDA2Nzg1OTY0OTY1ODIwM319LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjQ4LjYyMzgwOTgxNDQ1MzEyNSwibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuNzcxMzAxMjY5NTMxMjUsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NTAuMDgwNDQ4MTUwNjM0NzY2LCJsZWZ0IjoyMS44Mjc3NzAyMzMxNTQyOTcsIndpZHRoIjo1Ni44MjAyOTcyNDEyMTA5NCwiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo1MS41MzcwOTAzMDE1MTM2NywibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuODI2NTMwNDU2NTQyOTcsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NTMuMDEwNjY1ODkzNTU0NjksImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2Ljc2NTcyNDE4MjEyODkwNiwiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo1NC40NjczMDQyMjk3MzYzMywibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuNzkzNzEyNjE1OTY2OCwiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo1NS45MjM5NDYzODA2MTUyMzQsImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2Ljc5NTYxNjE0OTkwMjM0NCwiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo1Ny4zODA1ODg1MzE0OTQxNCwibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuNzc2OTI3OTQ3OTk4MDUsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NTguODU0MTY0MTIzNTM1MTU2LCJsZWZ0IjoyMS44Mjc3NzAyMzMxNTQyOTcsIndpZHRoIjo1Ni43ODA2NjYzNTEzMTgzNiwiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2MC4zMTA3OTg2NDUwMTk1MywibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuODMzMzc0MDIzNDM3NSwiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2MS43Njc0NDA3OTU4OTg0NCwibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuNzQ3MDA5Mjc3MzQzNzUsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NjMuMjI4MzEzNDQ2MDQ0OTIsImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2Ljc4ODEyNzg5OTE2OTkyLCJoZWlnaHQiOjEuNDA2Nzg1OTY0OTY1ODIwM319LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjY0LjcwMTg4OTAzODA4NTk0LCJsZWZ0IjoyMS44Mjc3NzAyMzMxNTQyOTcsIndpZHRoIjo1Ni43ODYyNjI1MTIyMDcwMywiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2Ni4xNTg1MzExODg5NjQ4NCwibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuODE1MzE5MDYxMjc5MywiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI1MDA1In19XQ%3D%3D. The negatively charged surfaces also inhibit biofilm formation on pond liners and biofilters, a common reservoir for opportunists like Aeromonas in fish. Scalable generators, optimized via perforated plate designs, maintain nanobubble concentrations (e.g., 1.04 × 10⁹ particles/mL) at neutral-alkaline pH, aligning with aquaculture conditions to maximize OH⁻ adsorption and antimicrobial potency (Wu et al., 2021; Zhu et al., 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjMwLCJzdWJ0eXBlIjp7fSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSmlhamlhIiwiZmFtaWx5IjoiV3UifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiS2VqaWEiLCJmYW1pbHkiOiJaaGFuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJDaGVuZyIsImZhbWlseSI6IkNlbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJYaWFvZ2FuZyIsImZhbWlseSI6Ild1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlJ1eWluIiwiZmFtaWx5IjoiTWFvIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Illpbmd5aW5nIiwiZmFtaWx5IjoiWmhlbmcifX1dLCJpZHMiOnsiaXNzbiI6WyIyMTkxLTA4NTUiXSwiaXNibiI6W10sImRvaSI6IjEwLjExODYvczEzNTY4LTAyMS0wMTI1NC0wIiwibWFnIjoiMzE3NTY4MzU2MSIsInBtaWQiOiIzNDE4NDEzNyIsInBtY2lkIjoiODIzOTEwOSIsIm9wZW5hbGV4IjoiVzMxNzU2ODM1NjEifSwidGl0bGUiOiJSb2xlIG9mIGJ1bGsgbmFub2J1YmJsZXMgaW4gcmVtb3Zpbmcgb3JnYW5pYyBwb2xsdXRhbnRzIGluIHdhc3Rld2F0ZXIgdHJlYXRtZW50IiwiYWJzdHJhY3QiOiJBYnN0cmFjdCBUaGUgb2NjdXJyZW5jZSBvZiBhIHZhcmlldHkgb2Ygb3JnYW5pYyBwb2xsdXRhbnRzIGhhcyBjb21wbGljYXRlZCB3YXN0ZXdhdGVyIHRyZWF0bWVudDsgdGh1cywgdGhlIHNlYXJjaCBmb3Igc3VzdGFpbmFibGUgYW5kIGVmZmVjdGl2ZSB0cmVhdG1lbnQgdGVjaG5vbG9neSBoYXMgZHJhd24gc2lnbmlmaWNhbnQgYXR0ZW50aW9uLiBJbiByZWNlbnQgeWVhcnMsIGJ1bGsgbmFub2J1YmJsZXMsIHdoaWNoIGhhdmUgZXh0cmFvcmRpbmFyeSBwcm9wZXJ0aWVzIGRpZmZlcmluZyBmcm9tIHRob3NlIG9mIG1pY3JvYnViYmxlcywgaW5jbHVkaW5nIGhpZ2ggc3RhYmlsaXR5IGFuZCBsb25nIHJlc2lkZW5jZSB0aW1lcyBpbiB3YXRlciwgbGFyZ2Ugc3BlY2lmaWMgc3VyZmFjZSBhcmVhcywgaGlnaCBnYXMgdHJhbnNmZXIgZWZmaWNpZW5jeSBhbmQgaW50ZXJmYWNlIHBvdGVudGlhbCwgYW5kIHRoZSBjYXBhYmlsaXR5IHRvIGdlbmVyYXRlIGZyZWUgcmFkaWNhbHMsIGhhdmUgc2hvd24gYXR0cmFjdGl2ZSB0ZWNobm9sb2dpY2FsIGFkdmFudGFnZXMgYW5kIHByb21pc2luZyBhcHBsaWNhdGlvbiBwcm9zcGVjdHMgZm9yIHdhc3Rld2F0ZXIgdHJlYXRtZW50LiBJbiB0aGlzIHJldmlldywgdGhlIGJhc2ljIGNoYXJhY3RlcmlzdGljcyBvZiBidWxrIG5hbm9idWJibGVzIGFyZSBzdW1tYXJpemVkIGluIGRldGFpbCwgYW5kIHJlY2VudCBmaW5kaW5ncyByZWxhdGVkIHRvIHRoZWlyIGltcGxlbWVudGF0aW9uIHBhdGh3YXlzIGFuZCBtZWNoYW5pc21zIGluIG9yZ2FuaWMgd2FzdGV3YXRlciB0cmVhdG1lbnQgYXJlIHN5c3RlbWF0aWNhbGx5IGRpc2N1c3NlZCwgd2hpY2ggaW5jbHVkZXMgaW1wcm92aW5nIHRoZSBhaXIgZmxvdGF0aW9uIHByb2Nlc3MsIGluY3JlYXNpbmcgd2F0ZXIgYWVyYXRpb24gdG8gcHJvbW90ZSBhZXJvYmljIGJpb2xvZ2ljYWwgdGVjaG5vbG9naWVzIGluY2x1ZGluZyBiaW9sb2dpY2FsIGFjdGl2YXRlZCBjYXJib24sIGFjdGl2YXRlZCBzbHVkZ2UsIGFuZCBtZW1icmFuZSBiaW9yZWFjdG9ycywgYW5kIGdlbmVyYXRpbmcgYWN0aXZlIGZyZWUgcmFkaWNhbHMgdGhhdCBveGlkaXNlIG9yZ2FuaWMgY29tcG91bmRzLiBGaW5hbGx5LCB0aGUgY3VycmVudCB0ZWNobm9sb2dpY2FsIGRpZmZpY3VsdGllcyBvZiBidWxrIG5hbm9idWJibGVzIGFyZSBhbmFseXNlZCwgYW5kIGZ1dHVyZSBmb2N1cyBhcmVhcyBmb3IgcmVzZWFyY2ggb24gYnVsayBuYW5vYnViYmxlIHRlY2hub2xvZ3kgYXJlIGFsc28gcHJvcG9zZWQuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjExODYvczEzNTY4LTAyMS0wMTI1NC0wIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2FtYi1leHByZXNzLnNwcmluZ2Vyb3Blbi5jb20vY291bnRlci9wZGYvMTAuMTE4Ni9zMTM1NjgtMDIxLTAxMjU0LTAiLCJpc3N1ZWREYXRlIjoiMjAyMS0wNi0yOCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkFNQiBFeHByZXNzIiwiaXNzdWUiOjEsInZvbHVtZSI6MTEsImltcGFjdEZhY3RvciI6IjQuMjYifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiU3ByaW5nZXIgTmF0dXJlIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo3NiwiY2l0ZWRCeUNvdW50Ijo3NX19LCJxdW90ZSI6IlRoZSBtb2RlbCBzdWNjZXNzZnVsbHkgZWx1Y2lkYXRlcyByZXB1bHNpdmUgZm9yY2VzICBiZXR3ZWVuIE9I4oiSIG9uIHRoZSBzdXJmYWNlIGFyZSBzdWZmaWNpZW50IHRvIHVsdGktbWF0ZWx5IGJhbGFuY2UgdGhlIHN1cmZhY2UgdGVuc2lvbiBmb3JjZXMgYXQgbmFub2J1YmJsZSAgc2l6ZXMsIHRodXMsIG1ha2luZyBuYW5vYnViYmxlcyBleGhpYml0IGdyZWF0IHN0YWJpbC1pdHkuIEFzIHNob3duIGluIHByZXZpb3VzIGxpdGVyYXR1cmVzLCB6ZXRhIHBvdGVudGlhbCB2YWwtdWVzIGNoYW5nZSB3aXRoIHRoZSBwSCBvZiB0aGUgbWVkaXVtIHZhcnksIGV4aGliaXRpbmcgIGEgdGVuZGVuY3kgdGhhdCB0aGUgbmVnYXRpdml0eSBvZiB6ZXRhIHBvdGVudGlhbCBpbmNyZWFzZXMgIHdpdGggc29sdXRpb24gcEggaW5jcmVhc2Ugd2hpY2ggcmVzdWx0cyBpbiBncmVhdGVyIGJ1Yi1ibGUgc3VyZmFjZSBjaGFyZ2UgYW5kIG1ha2UgYnViYmxlcyBtb3JlIHN0YWJsZSBkdWUgdG8gIGVsZWN0cm9zdGF0aWMgcmVwdWxzaW9uIChNZWVnb2RhIGV0IGFsLiAyMDE4KS4gTmFub2J1Yi1ibGVzIGdlbmVyYWxseSBzaG93IGhpZ2ggbmVnYXRpdmUgemV0YSBwb3RlbnRpYWwgb3dpbmcgdG8gIHRoZSBleGNlc3Mgb2YgaHlkcm94aWRlIGlvbnMgKE9I4oiSKSByZWxhdGl2ZSB0byBoeWRyb2dlbiAgaW9ucyAoSCspIGF0IHRoZSBnYXPigJN3YXRlciBpbnRlcmZhY2Ugb2YgYnViYmxlcyB1bmRlciAgd2lkZSByYW5nZSBvZiBzb2x1dGlvbiBwSCwgYnV0IG5hbm9idWJibGVzIGZvcm1lZCBpbiAgYWNpZGljIGVudmlyb25tZW50cyBhbHdheXMgc2hvdyBsb3dlciBzdGFiaWxpdHkgY29tLXBhcmVkIHdpdGggaW4gbmV1dHJhbCBvciBhbGthbGluZSBjb25kaXRpb25zIGJlY2F1c2UgYSAgbGFjayBvZiBPSOKIkiBhcmUgcmVxdWlyZWQgdG8gc3RhYmlsaXplIHRoZSBlbGVjdHJpYyBkb3VibGUgIGxheWVyIChOaXJtYWxrYXIgZXQgYWwuIDIwMTgpLCB0aGUgemV0YSBwb3RlbnRpYWwgb2YgbmFuby1idWJibGVzIGdlbmVyYXRlZCBpbiBzdHJvbmcgYWNpZGljIG1lZGl1bSBiZWxvdyBwSCAzICB0ZW5kcyB0byBiZSBwb3NpdGl2ZSBhbmQgbG93IHdoaWNoIGNhdXNlcyBoaWdoZXIgcG9zc2ktYmlsaXR5IGZvciBidWJibGUgY29hbGVzY2VuY2UgKFpoYW5nIGV0IGFsLiAyMDIwKS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjoxMy45NjgzMDU1ODc3Njg1NTUsImxlZnQiOjUyLjUzNDM0NzUzNDE3OTY5LCJ3aWR0aCI6MzguMzI2OTE1NzQwOTY2OCwiaGVpZ2h0IjoxLjgzNzY1NjYxNzE2NDYxMTh9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjoxNS40NzczMzQwMjI1MjE5NzMsImxlZnQiOjUxLjE5MDk3MTM3NDUxMTcyLCJ3aWR0aCI6MzkuMjg1MTYzODc5Mzk0NTMsImhlaWdodCI6MS44NDY1NjE1NTEwOTQwNTUyfX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MTcuMDAzNTYyOTI3MjQ2MDk0LCJsZWZ0Ijo1MS4xOTA0MzczMTY4OTQ1Mywid2lkdGgiOjM5LjY2OTEwNTUyOTc4NTE1NiwiaGVpZ2h0IjoxLjgzNzY1NjYxNzE2NDYxMTh9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjoxOC41MjAyNzcwMjMzMTU0MywibGVmdCI6NTEuMTkwNDM3MzE2ODk0NTMsIndpZHRoIjozOS4yODU2OTQxMjIzMTQ0NSwiaGVpZ2h0IjoxLjgzODI2NjI1MzQ3MTM3NDV9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjoyMC4wMzgyMTE4MjI1MDk3NjYsImxlZnQiOjUxLjE5MDQzNzMxNjg5NDUzLCJ3aWR0aCI6MzkuMjg1Njk0MTIyMzE0NDUsImhlaWdodCI6MS44Mzc2NTY2MTcxNjQ2MTE4fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MjEuNTU1NTM2MjcwMTQxNiwibGVmdCI6NTEuMTkwNDM3MzE2ODk0NTMsIndpZHRoIjozOS42NzUyMTY2NzQ4MDQ2OSwiaGVpZ2h0IjoxLjgzNzY1NjYxNzE2NDYxMTh9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjoyMy4wNzIyNDg0NTg4NjIzMDUsImxlZnQiOjUxLjE5MDQzNzMxNjg5NDUzLCJ3aWR0aCI6MzkuNjcwMzI2MjMyOTEwMTU2LCJoZWlnaHQiOjEuODM3NjU2NjE3MTY0NjExOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjI0LjU4ODk2MjU1NDkzMTY0LCJsZWZ0Ijo1MS4xOTA0MzczMTY4OTQ1Mywid2lkdGgiOjM5LjI4NTY5NDEyMjMxNDQ1LCJoZWlnaHQiOjEuODM4ODc1ODg5Nzc4MTM3Mn19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjI2LjEwNzUwNzcwNTY4ODQ3NywibGVmdCI6NTEuMTkwNDM3MzE2ODk0NTMsIndpZHRoIjozOS42NzIyNTI2NTUwMjkzLCJoZWlnaHQiOjEuODM3NjU2NjE3MTY0NjExOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjI3LjYyNDIyMTgwMTc1NzgxMiwibGVmdCI6NTEuMTkwNDM3MzE2ODk0NTMsIndpZHRoIjozOS4yODU2OTQxMjIzMTQ0NSwiaGVpZ2h0IjoxLjgzODI2NjI1MzQ3MTM3NDV9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjoyOS4xNDIxNTY2MDA5NTIxNSwibGVmdCI6NTEuMTkwNDM3MzE2ODk0NTMsIndpZHRoIjozOS42NzUyMzU3NDgyOTEwMTYsImhlaWdodCI6MS44Mzc2NTY2MTcxNjQ2MTE4fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MzAuNjUwNTczNzMwNDY4NzUsImxlZnQiOjUxLjE5MDQzNzMxNjg5NDUzLCJ3aWR0aCI6MzkuNjcxNzI2MjI2ODA2NjQsImhlaWdodCI6MS44NDY1NTg4MDkyODAzOTU1fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MzIuMTY3OTAwMDg1NDQ5MjIsImxlZnQiOjUxLjE4OTg5OTQ0NDU4MDA4LCJ3aWR0aCI6MzkuNjY5OTQwOTQ4NDg2MzMsImhlaWdodCI6MS44NDY1NTg4MDkyODAzOTU1fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MzMuNjkzNTE1Nzc3NTg3ODksImxlZnQiOjUxLjE5MDg2MDc0ODI5MTAxNiwid2lkdGgiOjM5LjY3NTcyMDIxNDg0Mzc1LCJoZWlnaHQiOjEuODM3NjU0NTkwNjA2Njg5NX19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjM1LjIxMDIzNTU5NTcwMzEyNSwibGVmdCI6NTEuMTkwODYwNzQ4MjkxMDE2LCJ3aWR0aCI6MzkuMjg1MjcwNjkwOTE3OTcsImhlaWdodCI6MS44Mzg4NzQxMDE2Mzg3OTR9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjozNi43Mjg3Nzg4MzkxMTEzMywibGVmdCI6NTEuMTkwNDM3MzE2ODk0NTMsIndpZHRoIjozOS42Njg5NzIwMTUzODA4NiwiaGVpZ2h0IjoxLjgzNzY1NDU5MDYwNjY4OTV9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjozOC4yMzcxNzg4MDI0OTAyMzQsImxlZnQiOjUxLjE5MDQzNzMxNjg5NDUzLCJ3aWR0aCI6MzkuNjY4ODA3OTgzMzk4NDQsImhlaWdodCI6MS44NDY1ODAwMjg1MzM5MzU1fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MzkuNzYyODIxMTk3NTA5NzY2LCJsZWZ0Ijo1MS4xODk5NTI4NTAzNDE4LCJ3aWR0aCI6MzkuMjg2MTgyNDAzNTY0NDUsImhlaWdodCI6MS44MzgyNjA1MzE0MjU0NzZ9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo0MS4yODA3NTAyNzQ2NTgyLCJsZWZ0Ijo1MS4xOTA0MzczMTY4OTQ1Mywid2lkdGgiOjM5LjY3MjczMzMwNjg4NDc2NiwiaGVpZ2h0IjoxLjgzNzY1NDU5MDYwNjY4OTV9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo0Mi43OTc0NzAwOTI3NzM0NCwibGVmdCI6NTEuMTkwNDM3MzE2ODk0NTMsIndpZHRoIjozOS4yODU2OTQxMjIzMTQ0NSwiaGVpZ2h0IjoxLjgzODI2MDUzMTQyNTQ3Nn19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjQ0LjMxNTM5OTE2OTkyMTg3NSwibGVmdCI6NTEuMTkwNDM3MzE2ODk0NTMsIndpZHRoIjozOS4yODU2OTQxMjIzMTQ0NSwiaGVpZ2h0IjoxLjgzNzY1NDU5MDYwNjY4OTV9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIzIn19LHsib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5pbmciLCJmYW1pbHkiOiJaaHUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTWluLVl1IiwiZmFtaWx5IjoiTGkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiS29oZWkiLCJmYW1pbHkiOiJTaGliYXRhIn19XSwiaWRzIjp7Imlzc24iOltdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMzM5MC9lbmdwcm9jMjAyMzA1NTAyMyIsIm9wZW5hbGV4IjoiVzQzODkxNDU1NzgiLCJleGEiOiJodHRwczovL3d3dy5tZHBpLmNvbS8yNjczLTQ1OTEvNTUvMS8yMyJ9LCJ0aXRsZSI6IlRoZSBEZXZlbG9wbWVudCBvZiBhIEhpZ2gtQ29uY2VudHJhdGlvbiBPeHlnZW5hdGVkIFdhdGVyIEdlbmVyYXRvciBCYXNlZCBvbiBOYW5vYnViYmxlcyBhbmQgSXRzIEFwcGxpY2F0aW9uIiwiYWJzdHJhY3QiOiJXYXRlciB3aXRoIGEgaGlnaCBjb25jZW50cmF0aW9uIG9mIG94eWdlbiBpcyBuZWVkZWQgZm9yIHRoZSBhcXVhY3VsdHVyZSBpbmR1c3RyeSBpbiBKYXBhbi4gSW4gdGhlIGN1cnJlbnQgc3R1ZHksIHRoZSBwcmVzc3VyaXplZCBkaXNzb2x1dGlvbiBtZXRob2Qgd2FzIGVtcGxveWVkIHRvIGdlbmVyYXRlIGhpZ2gtY29uY2VudHJhdGlvbiBveHlnZW5hdGVkIHdhdGVyIChIQ09XKSBieSBwcm9kdWNpbmcgb3h5Z2VuIG5hbm9idWJibGVzIGluIHRoZSB3YXRlci4gSW4gb3JkZXIgdG8gaW52ZXN0aWdhdGUgZmFjdG9ycyBzdWNoIGFzIHRlbXBlcmF0dXJlLCBnZW9tZXRyaWMgY29uZGl0aW9ucywgYW5kIHRoZWlyIGluZmx1ZW5jZSBvbiB0aGUgb3h5Z2VuIGNvbmNlbnRyYXRpb24sIGEgc3BlY2lhbCBuYW5vYnViYmxlIGdlbmVyYXRvciB3YXMgaW1wcm92ZWQgYnkgY2hhbmdpbmcgdGhlIG51bWJlciBhbmQgdGhlIGRpYW1ldGVyIG9mIHRoZSBob2xlcyBvZiB0aGUgcGVyZm9yYXRlZCBwbGF0ZSBpbiB0aGlzIHN0dWR5LiBUaGVuLCBhbiBleHBlcmltZW50YWwgc3lzdGVtIHdoZXJlIG94eWdlbiBhbmQgd2F0ZXIgd2VyZSBzZXBhcmF0ZWx5IGludHJvZHVjZWQgaW5zaWRlIHRoZSBwcm9wb3NlZCBuYW5vYnViYmxlIGdlbmVyYXRvciB3YXMgZGVzaWduZWQuIFRoZSBkaXNzb2x2ZWQgb3h5Z2VuIGNvbmNlbnRyYXRpb24gd2FzIG1lYXN1cmVkIHVuZGVyIGRpZmZlcmVudCBjb25kaXRpb25zLiBGaW5hbGx5LCB0aGUgcHJvZHVjZWQgSENPVyB3YXMgdXNlZCB0byBjdWx0aXZhdGUgYSBtaW5pLXN1bmZsb3dlci4gVGhyb3VnaCBhIHNlcmllcyBvZiBleHBlcmltZW50cywgaXQgd2FzIGZvdW5kIHRoYXQgd2l0aCB0aGUgaW1wcm92ZWQgcGVyZm9yYXRlZCBwbGF0ZSwgdGhlIGRpc3NvbHZlZCBveHlnZW4gY29uY2VudHJhdGlvbiB3YXMgaW5jcmVhc2VkIGFuZCB0aGUgbmFub2J1YmJsZSBnZW5lcmF0b3IgcmVhY2hlZCB0aGUgc2F0dXJhdGlvbiBzdGF0ZSBxdWlja2x5LCB3aGlsZSB0aGUgbWluaS1zdW5mbG93ZXIgY3VsdGl2YXRlZCB3aXRoIHRoZSBIQ09XIGFwcGVhcmVkIHRvIGdyb3cgbGFyZ2VyIHRoYW4gdGhhdCB3aXRoIHRhcCB3YXRlci4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMzM5MC9lbmdwcm9jMjAyMzA1NTAyMyIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cubWRwaS5jb20vMjY3My00NTkxLzU1LzEvMjMvcGRmP3ZlcnNpb249MTcwMTI0MzExOCIsImlzc3VlZERhdGUiOiIyMDIzLTExLTI5IiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInZvbHVtZSI6MzN9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjIzLCJsYXN0UGFnZSI6MjN9fSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjIsImNpdGVkQnlDb3VudCI6MX19LCJxdW90ZSI6IkFsdG1ldHJpY1tfc2hhcmVfIFNoYXJlXShodHRwczovL3d3dy5tZHBpLmNvbS8yNjczLTQ1OTEvNTUvMS8yMykgW19hbm5vdW5jZW1lbnRfIEhlbHBdKGh0dHBzOi8vd3d3Lm1kcGkuY29tLzI2NzMtNDU5MS81NS8xLzIzKSBbX2Zvcm1hdFxcX3F1b3RlXyBDaXRlXShqYXZhc2NyaXB0OnZvaWQoMCk7KSBbX3F1ZXN0aW9uXFxfYW5zd2VyXyBEaXNjdXNzIGluIFNjaVByb2ZpbGVzXShodHRwczovL3NjaXByb2ZpbGVzLmNvbS9kaXNjdXNzaW9uLWdyb3Vwcy9wdWJsaWMvMTAuMzM5MC9lbmdwcm9jMjAyMzA1NTAyMz91dG1fc291cmNlPW1wZGkuY29tJnV0bV9tZWRpdW09cHVibGljYXRpb24mdXRtX2NhbXBhaWduPWRpc2N1c3NfaW5fc2NpcHJvZmlsZXMpIFdhdGVyIHdpdGggYSBoaWdoIGNvbmNlbnRyYXRpb24gb2Ygb3h5Z2VuIGlzIG5lZWRlZCBmb3IgdGhlIGFxdWFjdWx0dXJlIGluZHVzdHJ5IGluIEphcGFuLiBJbiB0aGUgY3VycmVudCBzdHVkeSwgdGhlIHByZXNzdXJpemVkIGRpc3NvbHV0aW9uIG1ldGhvZCB3YXMgZW1wbG95ZWQgdG8gZ2VuZXJhdGUgaGlnaC1jb25jZW50cmF0aW9uIG94eWdlbmF0ZWQgd2F0ZXIgKEhDT1cpIGJ5IHByb2R1Y2luZyBveHlnZW4gbmFub2J1YmJsZXMgaW4gdGhlIHdhdGVyLiBJbiBvcmRlciB0byBpbnZlc3RpZ2F0ZSBmYWN0b3JzIHN1Y2ggYXMgdGVtcGVyYXR1cmUsIGdlb21ldHJpYyBjb25kaXRpb25zLCBhbmQgdGhlaXIgaW5mbHVlbmNlIG9uIHRoZSBveHlnZW4gY29uY2VudHJhdGlvbiwgYSBzcGVjaWFsIG5hbm9idWJibGUgZ2VuZXJhdG9yIHdhcyBpbXByb3ZlZCBieSBjaGFuZ2luZyB0aGUgbnVtYmVyIGFuZCB0aGUgZGlhbWV0ZXIgb2YgdGhlIGhvbGVzIG9mIHRoZSBwZXJmb3JhdGVkIHBsYXRlIGluIHRoaXMgc3R1ZHkuIn1d. Economic feasibility is promising, with short payback periods (1.7 years) for integrated systems (Susanti et al., 2021)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkxpbHkiLCJmYW1pbHkiOiJTdXNhbnRpIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlN1eXVkIFdhcm5vIiwiZmFtaWx5IjoiVXRvbW8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTm92ZXJpdGEgRGlhbiIsImZhbWlseSI6IlRha2FyaW5hIn19XSwiaWRzIjp7Imlzc24iOlsiMjExNy00NDU4IiwiMjI3My0xNzA5Il0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDUxL2Jpb2NvbmYvMjAyMTMzMDUwMDUiLCJtYWciOiIzMTkzNTYzNzAzIiwib3BlbmFsZXgiOiJXMzE5MzU2MzcwMyJ9LCJ0aXRsZSI6IlN1c3RhaW5hYmlsaXR5IGFuZCBmZWFzaWJpbGl0eSBhc3Nlc3NtZW50cyBvZiBuYW5vYnViYmxlIGFlcmF0aW9uIHRlY2hub2xvZ3kgaW4gZWNvbm9taWMtc29jaW8gZW52aXJvbm1lbnQgb2YgUGVuYWV1cyB2YW5uYW1laSBzaHJpbXAgZmFybWluZyIsImFic3RyYWN0IjoiUGVuYWV1cyB2YW5uYW1laSBzaHJpbXAgZmFybWluZyBpbiBJbmRvbmVzaWEgaXMgZmFjZWQgd2l0aCBzZXZlcmFsIGNoYWxsZW5nZXMgaW4gdGhlIGVudmlyb25tZW50YWwsIGVjb25vbWljLCBhbmQgc29jaWFsIGFzcGVjdHMuIFRoZXJlZm9yZSwgdGhpcyBzdHVkeSBhaW1zIHRvIGFzc2VzcyB0aGUgYmVuZWZpdHMgb2Ygbm92ZWwgbmFub2J1YmJsZSBhZXJhdGlvbiBzeXN0ZW1zIGZvciB0aGUgZWNvc3lzdGVtLCBidXNpbmVzc2VzLCBhbmQgY29tbXVuaXRpZXMuIFRoaXMgaXMgYW4gZXhwZXJpbWVudGFsIHN0dWR5IGNvbmR1Y3RlZCBieSBjb21wYXJpbmcgUC4gdmFubmFtZWkgcG9zdCBsYXJ2YSAxMCByZWFyZWQgYXQgYSBkZW5zaXR5IG9mIDIwMDAgaW5kcy4vTCBpbiBhIHBvbmQgdHJlYXRlZCB3aXRoIG5hbm9idWJibGUgYW5kIHdpdGhvdXQgdHJlYXRtZW50cyAoY29udHJvbCkuIEZ1cnRoZXJtb3JlLCB0aGUgTmV0IFByZXNlbnQgVmFsdWUgKE5QVikgYW5kIHBheWJhY2sgcGVyaW9kIHdlcmUgY2FsY3VsYXRlZCB0byBhc3Nlc3MgdGhlIGVjb25vbWljIGZlYXNpYmlsaXR5IG9mIG5hbm9idWJibGUsIHdoaWxlIGNvbW11bml0eSBpbnRlcnZpZXdzIHdlcmUgdXNlZCBmb3IgdGhlIHNvY2lhbCBzZWN0b3JzLiBCYXNlZCBvbiB0aGUgcmVzdWx0cywgZW52aXJvbm1lbnRhbCB3YXN0ZXMgb2Ygc2hyaW1wIGZhcm1pbmcgaW4gZm9ybSBvZiB0b3RhbCBhbW1vbmlhIG5pdHJvZ2VuIChUQU4pIHdlcmUgcmVkdWNlZCBieSA5JSBmcm9tIDIuNTggbWcvbCAoOTUlQ0k6IDAsOTEsIDQsMjUpIGluIGNvbnRyb2wgY29tcGFyZWQgdG8gMi4zNSBtZy9sICg5NSVDSTogMCw4NiwgMyw4NCkgaW4gdHJlYXRtZW50LiBGdXJ0aGVybW9yZSwgdGhlIHJldmVudWVzIGZyb20gcG9zdCBsYXJ2YSBzYWxlcyBmb3IgZml2ZSB5ZWFycyBhbmQgbmFub2J1YmJsZSBpbnZlc3RtZW50IGNvc3RzIHNob3dlZCB0aGF0IHRoZSBlc3RpbWF0ZWQgTlBWIHdhcyBJRFIgNjQsODI0LDM3NCB3aXRoIGEgcGF5YmFjayBwZXJpb2Qgb2YgMS43IHllYXJzLiBUaGUgaW50ZXJ2aWV3cyBvbiB0cmFkaXRpb25hbCBzaHJpbXAgZmFybWluZyBzaG93ZWQgdGhhdCA2MS4xJSB0byA3Mi4yJSBvZiBjb21tdW5pdHkgbWVtYmVycyBhZ3JlZWQgb24gdGhlIHVzZSBvZiBuYW5vYnViYmxlIGFlcmF0aW9uIHRvIHN1cHBvcnQgbGl2ZWxpaG9vZHMuIFRoZXJlZm9yZSwgdGhlIHVzZSBvZiBuYW5vYnViYmxlcyBpcyBmZWFzaWJsZSB0byBzdXBwb3J0IHN1c3RhaW5hYmxlIFAuIHZhbm5hbWVpIGZhcm1pbmcuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjEwNTEvYmlvY29uZi8yMDIxMzMwNTAwNSIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cuYmlvLWNvbmZlcmVuY2VzLm9yZy9hcnRpY2xlcy9iaW9jb25mL3BkZi8yMDIxLzA1L2Jpb2NvbmZfaWNhdmVzczIwMjFfMDUwMDUucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjEtMDEtMDEiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA5IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJCSU8gV2ViIG9mIENvbmZlcmVuY2VzIiwidm9sdW1lIjozMywiaW1wYWN0RmFjdG9yIjoiMC42MiJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjUwMDUsImxhc3RQYWdlIjo1MDA1fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkVEUCBTY2llbmNlcyJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MTEsImNpdGVkQnlDb3VudCI6NH19LCJxdW90ZSI6IkFic3RyYWN0LiBQZW5hZXVzIHZhbm5hbWVpIHNocmltcCBmYXJtaW5nIGluIEluZG9uZXNpYSBpcyBmYWNlZCB3aXRoIHNldmVyYWwgY2hhbGxlbmdlcyBpbiB0aGUgZW52aXJvbm1lbnRhbCwgZWNvbm9taWMsIGFuZCBzb2NpYWwgYXNwZWN0cy4gIFRoZXJlZm9yZSwgdGhpcyBzdHVkeSBhaW1zIHRvIGFzc2VzcyB0aGUgYmVuZWZpdHMgb2Ygbm92ZWwgbmFub2J1YmJsZSAgYWVyYXRpb24gc3lzdGVtcyBmb3IgdGhlIGVjb3N5c3RlbSwgYnVzaW5lc3NlcywgYW5kIGNvbW11bml0aWVzLiBUaGlzIGlzIGFuICBleHBlcmltZW50YWwgc3R1ZHkgY29uZHVjdGVkIGJ5IGNvbXBhcmluZyBQLiB2YW5uYW1laSBwb3N0IGxhcnZhIDEwICByZWFyZWQgYXQgYSBkZW5zaXR5IG9mIDIwMDAgaW5kcy4vTCBpbiBhIHBvbmQgdHJlYXRlZCB3aXRoIG5hbm9idWJibGUgYW5kICB3aXRob3V0IHRyZWF0bWVudHMgKGNvbnRyb2wpLiBGdXJ0aGVybW9yZSwgdGhlIE5ldCBQcmVzZW50IFZhbHVlIChOUFYpIGFuZCAgcGF5YmFjayBwZXJpb2Qgd2VyZSBjYWxjdWxhdGVkIHRvIGFzc2VzcyB0aGUgZWNvbm9taWMgZmVhc2liaWxpdHkgb2YgIG5hbm9idWJibGUsIHdoaWxlIGNvbW11bml0eSBpbnRlcnZpZXdzIHdlcmUgdXNlZCBmb3IgdGhlIHNvY2lhbCBzZWN0b3JzLiAgQmFzZWQgb24gdGhlIHJlc3VsdHMsIGVudmlyb25tZW50YWwgd2FzdGVzIG9mIHNocmltcCBmYXJtaW5nIGluIGZvcm0gb2YgIHRvdGFsIGFtbW9uaWEgbml0cm9nZW4gKFRBTikgd2VyZSByZWR1Y2VkIGJ5IDklIGZyb20gMi41OCBtZy9sICg5NSVDSTogIDAsOTEsIDQsMjUpIGluIGNvbnRyb2wgY29tcGFyZWQgdG8gMi4zNSBtZy9sICg5NSVDSTogMCw4NiwgMyw4NCkgaW4gIHRyZWF0bWVudC4gRnVydGhlcm1vcmUsIHRoZSByZXZlbnVlcyBmcm9tIHBvc3QgbGFydmEgc2FsZXMgZm9yIGZpdmUgeWVhcnMgYW5kICBuYW5vYnViYmxlIGludmVzdG1lbnQgY29zdHMgc2hvd2VkIHRoYXQgdGhlIGVzdGltYXRlZCBOUFYgd2FzIElEUiAgNjQsODI0LDM3NCB3aXRoIGEgcGF5YmFjayBwZXJpb2Qgb2YgMS43IHllYXJzLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjQ1LjQ4MzI4NzgxMTI3OTMsImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2LjgwMDU2NzYyNjk1MzEyNSwiaGVpZ2h0IjoxLjc0OTIzODYxMDI2NzYzOTJ9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo0Ny4xNjcxNzE0NzgyNzE0ODQsImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2Ljc3NTAzOTY3Mjg1MTU2LCJoZWlnaHQiOjEuNDA2Nzg1OTY0OTY1ODIwM319LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjQ4LjYyMzgwOTgxNDQ1MzEyNSwibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuNzcxMzAxMjY5NTMxMjUsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NTAuMDgwNDQ4MTUwNjM0NzY2LCJsZWZ0IjoyMS44Mjc3NzAyMzMxNTQyOTcsIndpZHRoIjo1Ni44MjAyOTcyNDEyMTA5NCwiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo1MS41MzcwOTAzMDE1MTM2NywibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuODI2NTMwNDU2NTQyOTcsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NTMuMDEwNjY1ODkzNTU0NjksImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2Ljc2NTcyNDE4MjEyODkwNiwiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo1NC40NjczMDQyMjk3MzYzMywibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuNzkzNzEyNjE1OTY2OCwiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo1NS45MjM5NDYzODA2MTUyMzQsImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2Ljc5NTYxNjE0OTkwMjM0NCwiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo1Ny4zODA1ODg1MzE0OTQxNCwibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuNzc2OTI3OTQ3OTk4MDUsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NTguODU0MTY0MTIzNTM1MTU2LCJsZWZ0IjoyMS44Mjc3NzAyMzMxNTQyOTcsIndpZHRoIjo1Ni43ODA2NjYzNTEzMTgzNiwiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2MC4zMTA3OTg2NDUwMTk1MywibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuODMzMzc0MDIzNDM3NSwiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2MS43Njc0NDA3OTU4OTg0NCwibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuNzQ3MDA5Mjc3MzQzNzUsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NjMuMjI4MzEzNDQ2MDQ0OTIsImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2Ljc4ODEyNzg5OTE2OTkyLCJoZWlnaHQiOjEuNDA2Nzg1OTY0OTY1ODIwM319LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjY0LjcwMTg4OTAzODA4NTk0LCJsZWZ0IjoyMS44Mjc3NzAyMzMxNTQyOTcsIndpZHRoIjo1Ni43ODYyNjI1MTIyMDcwMywiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2Ni4xNTg1MzExODg5NjQ4NCwibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuODE1MzE5MDYxMjc5MywiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI1MDA1In19XQ%3D%3D.

In summary, nanobubbles revolutionize aquaculture disease control through ROS-driven disinfection, biophysical disruption, and community modulation, outperforming antibiotics in efficacy and sustainability. Their chemical-free nature minimizes resistance risks and regulatory hurdles, fostering resilient production amid intensifying biosecurity demands (Jia et al., 2023; Svetovoy, 2021)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjMwLCJzdWJ0eXBlIjp7fSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTWluZ3lpIiwiZmFtaWx5IjoiSmlhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik11aGFtbWFkIFVzbWFuIiwiZmFtaWx5IjoiRmFyaWQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSmVoYWQgQS4iLCJmYW1pbHkiOiJLaGFycmF6In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hbGxhcGFuZW5pIE1hbm9qIiwiZmFtaWx5IjoiS3VtYXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2hhdWhyYXQgUy4iLCJmYW1pbHkiOiJDaG9wcmEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW0iLCJmYW1pbHkiOiJKYW5nIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlkuTS4gSm9obiIsImZhbWlseSI6IkNoZXcifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2FtaXIgS3VtYXIiLCJmYW1pbHkiOiJLaGFuYWwifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiR3VhbmdoYW8iLCJmYW1pbHkiOiJDaGVuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFsaWNpYSBLeW91bmdqaW4iLCJmYW1pbHkiOiJBbiJ9fV0sImlkcyI6eyJpc3NuIjpbIjAwNDMtMTM1NCIsIjE4NzktMjQ0OCJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMTAxNi9qLndhdHJlcy4yMDIzLjEyMDYxMyIsInBtaWQiOiIzNzczODk0MCIsIm9wZW5hbGV4IjoiVzQzODY2MTg0NTkifSwidGl0bGUiOiJOYW5vYnViYmxlcyBpbiB3YXRlciBhbmQgd2FzdGV3YXRlciB0cmVhdG1lbnQgc3lzdGVtczogU21hbGwgYnViYmxlcyBtYWtpbmcgYmlnIGRpZmZlcmVuY2UiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAxNi9qLndhdHJlcy4yMDIzLjEyMDYxMyIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9wdXJlaG9zdC5iYXRoLmFjLnVrL3dzL2ZpbGVzLzI5NjM0MzkwMS9OYW5vYnViYmxlc19wcmVfcHJvb2YucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjMtMDktMTEiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA4IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJXYXRlciBSZXNlYXJjaCIsInZvbHVtZSI6MjQ1LCJpbXBhY3RGYWN0b3IiOiIxMy40NCJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjEyMDYxMywibGFzdFBhZ2UiOjEyMDYxM319LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJFbHNldmllciBCViJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MTExLCJjaXRlZEJ5Q291bnQiOjc1fX0sInF1b3RlIjoiU2V2ZXJhbCBzdHVkaWVzIGhhdmUgcHJvcG9zZWQgdGhhdCB0aGUgbmVnYXRpdmUgY2hhcmdlIG9mIE5CcyBhcmlzZXMgZHVlIHRvIHRoZWlyIHNtYWxsIHJhZGl1cyBhbmQgdGhlIGhpZ2ggcG9sYXJpdHkgb2YgaHlkcm94eWwgaW9ucywgd2hpY2ggbGVhZCB0byB0aGUgYWRzb3JwdGlvbiBvZiBkaXNzb2NpYXRlZCBoeWRyb3h5bCBpb25zIG9uIHRoZSBidWJibGUgc3VyZmFjZSwgd2hpbGUgaHlkcm9nZW4gaW9ucyBhcmUgZWFzaWx5IGh5ZHJhdGVkIGFuZCBwcmVmZXIgdG8gc3RheSBpbiB0aGUgYnVsayBsaXF1aWQuIEEgaGlnaGVyIGFic29sdXRlIHpldGEgcG90ZW50aWFsIGluZGljYXRlcyBoaWdoZXIgc3RhYmlsaXR5LCBhcyB0aGUgcmVwdWxzaXZlIGZvcmNlIGdlbmVyYXRlZCBieSBOQnPigJggc3VyZmFjZXMgcHJldmVudHMgdGhlbSBmcm9tIG1lcmdpbmcgd2l0aCBvbmUgYW5vdGhlciAoVXNoaWt1Ym8gZXQgYWwuLCAyMDEwKS4gMTEgU3VyZmFjZSBoeWRyb3Bob2JpY2l0eSBpcyBhbm90aGVyIGVzc2VudGlhbCBwcm9wZXJ0eSBvZiBOQnMgYXMgaXQgYWxsb3dzIE5CcyB0byBpbnRlcmFjdCB3aXRoIG90aGVyIHBhcnRpY2xlcyB0aHJvdWdoIGh5ZHJvcGhvYmljIGF0dHJhY3Rpb24uIEJOQnMgY2FuIGFkc29yYiBvbiBoeWRyb3Bob2JpYyBwYXJ0aWNsZXMgYW5kIGluY3JlYXNlIHRoZWlyIG1vYmlsaXR5IGluIHRoZSBsaXF1aWQsIHdoaWxlIGF0dGFjaGVkIE5CcyBjYW4gcHJvbW90ZSBwYXJ0aWNsZS1wYXJ0aWNsZSBvciBwYXJ0aWNsZS0gYnViYmxlIGludGVyYWN0aW9ucyB0aHJvdWdoIGNhcGlsbGFyeSBicmlkZ2luZyBlZmZlY3RzIHRoYXQgcHJvZHVjZSBhbiBhdHRyYWN0aXZlIGZvcmNlIGJldHdlZW4gdHdvIHN1cmZhY2VzIChIYW1wdG9uIGFuZCBOZ3V5ZW4sIDIwMTApLiAyLjEuMyBMb25nIHJlc2lkZW5jZSB0aW1lIGluIHdhdGVyOiBOQnPigJkgbG9uZy10ZXJtIHN0YWJpbGl0eSBBbW9uZyB0aGUgdmFyaW91cyBjaGFyYWN0ZXJpc3RpY3MgTkJzIHBvc3Nlc3MsIHRoZSBtb3N0IGFzdG9uaXNoaW5nIGlzIHRoZWlyIGxvbmdldml0eS4gVXBvbiBnZW5lcmF0aW9uLCBidWJibGVzIGdlbmVyYWxseSBtYWludGFpbiB0aGVpciBkaWFtZXRlcnMgZm9yIGEgY2VydGFpbiBhbW91bnQgb2YgdGltZSwgYWZ0ZXIgd2hpY2ggdGhlIGJ1YmJsZXMgYmVnaW4gdG8gZ3JvdyBhbmQgZXZlbnR1YWxseSBkaXNzb2x2ZSBpbiB0aGUgc29sdXRpb24uIEV4cGVyaW1lbnRhbCByZXN1bHRzIGhhdmUgY29uZmlybWVkIHRoYXQgTkJzIGFyZSBhYmxlIHRvIGV4aXN0IGluIHdhdGVyIGZyb20gZGF5cyB0byBtb250aHMuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxMSwiYmJveCI6eyJ0b3AiOjY4LjYzMTk2NTYzNzIwNzAzLCJsZWZ0Ijo5Ljc0MTE4MDQxOTkyMTg3NSwid2lkdGgiOjc3LjAxMDI1MzkwNjI1LCJoZWlnaHQiOjEuODQ5MjYzNjY4MDYwMzAyN319LHsicGFnZU51bWJlciI6MTEsImJib3giOnsidG9wIjo3MS44MjM4MjIwMjE0ODQzOCwibGVmdCI6OS43NDExODA0MTk5MjE4NzUsIndpZHRoIjo3Ny4wMDYzMDk1MDkyNzczNCwiaGVpZ2h0IjoxLjg0OTI2MzY2ODA2MDMwMjd9fSx7InBhZ2VOdW1iZXIiOjExLCJiYm94Ijp7InRvcCI6NzUuMDEyMjE0NjYwNjQ0NTMsImxlZnQiOjkuNzQxMTgwNDE5OTIxODc1LCJ3aWR0aCI6NzYuOTQ4Nzc2MjQ1MTE3MTksImhlaWdodCI6MS44NDkyNjM2NjgwNjAzMDI3fX0seyJwYWdlTnVtYmVyIjoxMSwiYmJveCI6eyJ0b3AiOjc4LjIwMDU5OTY3MDQxMDE2LCJsZWZ0Ijo5Ljc0MTE4MDQxOTkyMTg3NSwid2lkdGgiOjc3LjAwODI4NTUyMjQ2MDk0LCJoZWlnaHQiOjEuODQ5MjYzNjY4MDYwMzAyN319LHsicGFnZU51bWJlciI6MTEsImJib3giOnsidG9wIjo4MS4zODg5OTk5Mzg5NjQ4NCwibGVmdCI6OS43NDExODA0MTk5MjE4NzUsIndpZHRoIjo3Ny4wMTAyNDYyNzY4NTU0NywiaGVpZ2h0IjoxLjg0OTI2MzY2ODA2MDMwMjd9fSx7InBhZ2VOdW1iZXIiOjExLCJiYm94Ijp7InRvcCI6ODQuNTc2OTE5NTU1NjY0MDYsImxlZnQiOjkuNzQxMTgwNDE5OTIxODc1LCJ3aWR0aCI6MTkuNjE2NjI0ODMyMTUzMzIsImhlaWdodCI6MS44NDkyNjM2NjgwNjAzMDI3fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjg3LjQ4ODA1OTk5NzU1ODYsImxlZnQiOjQ3LjAxODI1MzMyNjQxNjAxNiwid2lkdGgiOjIuNDUyMTYxNzg4OTQwNDI5NywiaGVpZ2h0IjoxLjg0OTI2MzY2ODA2MDMwMjd9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MTEuNjQ4NzM4ODYxMDgzOTg0LCJsZWZ0Ijo5Ljc0MTE4MDQxOTkyMTg3NSwid2lkdGgiOjc2Ljk1MjcwNTM4MzMwMDc4LCJoZWlnaHQiOjEuODQ5MjY1NTc1NDA4OTM1NX19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoxNC44Mzk0MzQ2MjM3MTgyNjIsImxlZnQiOjkuNzQxMTgwNDE5OTIxODc1LCJ3aWR0aCI6NzYuOTQ4NzgzODc0NTExNzIsImhlaWdodCI6MS44NDkyNjU1NzU0MDg5MzU1fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjE4LjAyNzgzMzkzODU5ODYzMywibGVmdCI6OS43NDExODA0MTk5MjE4NzUsIndpZHRoIjo3Ni41MDYwNzI5OTgwNDY4OCwiaGVpZ2h0IjoxLjg0OTI2NTU3NTQwODkzNTV9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjEuMjE2MjE3MDQxMDE1NjI1LCJsZWZ0Ijo5Ljc0MTE4MDQxOTkyMTg3NSwid2lkdGgiOjc2Ljk4NTczMzAzMjIyNjU2LCJoZWlnaHQiOjEuODQ5MjY1NTc1NDA4OTM1NX19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyNC40MDQ2MDAxNDM0MzI2MTcsImxlZnQiOjkuNzQxMTgwNDE5OTIxODc1LCJ3aWR0aCI6MzUuMzc0MTk4OTEzNTc0MjIsImhlaWdodCI6MS44NDkyNjU1NzU0MDg5MzU1fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjMyLjIxMTMxMTM0MDMzMjAzLCJsZWZ0Ijo5Ljc0MTE4MDQxOTkyMTg3NSwid2lkdGgiOjUyLjQ2NDEyMjc3MjIxNjgsImhlaWdodCI6MS45MTk5NjUyNjcxODEzOTY1fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM2LjA1MjYyNzU2MzQ3NjU2LCJsZWZ0Ijo5Ljc0MTE4MDQxOTkyMTg3NSwid2lkdGgiOjc3LjAwODI4NTUyMjQ2MDk0LCJoZWlnaHQiOjEuODQ5MjY3NDgyNzU3NTY4NH19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozOS4yNDEwMTYzODc5Mzk0NSwibGVmdCI6OS43NDExODA0MTk5MjE4NzUsIndpZHRoIjo3Ny4wMDgyODU1MjI0NjA5NCwiaGVpZ2h0IjoxLjg0OTI2NzQ4Mjc1NzU2ODR9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDIuNDI5NDA5MDI3MDk5NjEsImxlZnQiOjkuNzQxMTgwNDE5OTIxODc1LCJ3aWR0aCI6NzcuMDAyNDAzMjU5Mjc3MzQsImhlaWdodCI6MS44NDkyNjc0ODI3NTc1Njg0fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQ1LjYxNzc5NDAzNjg2NTIzNCwibGVmdCI6OS43NDExODA0MTk5MjE4NzUsIndpZHRoIjo3Ny4wMTAyNDYyNzY4NTU0NywiaGVpZ2h0IjoxLjg0OTI2NzQ4Mjc1NzU2ODR9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIxMjA2MjQifX0seyJvcmlnaW4iOjQsIndvcmsiOnsidHlwZSI6OCwic3VidHlwZSI6eyJvbmVvZktpbmQiOiJhcnRpY2xlVHlwZSIsImFydGljbGVUeXBlIjoxfSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiVi4gQi4iLCJmYW1pbHkiOiJTdmV0b3ZveSJ9fV0sImlkcyI6eyJpc3NuIjpbIjEzNTktMDI5NCIsIjE4NzktMDM5OSJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMTAxNi9qLmNvY2lzLjIwMjEuMTAxNDIzIiwibWFnIjoiMzEyNjQ4NDg0OCIsIm9wZW5hbGV4IjoiVzMxMjY0ODQ4NDgifSwidGl0bGUiOiJTcG9udGFuZW91cyBjaGVtaWNhbCByZWFjdGlvbnMgYmV0d2VlbiBoeWRyb2dlbiBhbmQgb3h5Z2VuIGluIG5hbm9idWJibGVzIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjEwMTYvai5jb2Npcy4yMDIxLjEwMTQyMyIsInJlc291cmNlVXJsIjoiaHR0cDovL2FyeGl2Lm9yZy9wZGYvMjEwMi4wMzEyNiIsImlzc3VlZERhdGUiOiIyMDIxLTAyLTA1IiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiQ3VycmVudCBPcGluaW9uIGluIENvbGxvaWQgJiBJbnRlcmZhY2UgU2NpZW5jZSIsInZvbHVtZSI6NTJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjEwMTQyMywibGFzdFBhZ2UiOjEwMTQyM319LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJFbHNldmllciBCViJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NjEsImNpdGVkQnlDb3VudCI6Mn19LCJxdW90ZSI6IlRoaXMgYXR0ZW50aW9uIGlzIGRyaXZlbiBieSBhIGZ1bmRhLW1lbnRhbCBwcm9ibGVtOiBOQnMgbGl2ZSBtdWNoIHRvbyBsb25nIGluIGNvbXBhcmktc29uIHdpdGggdGhlIHByZWRpY3Rpb24gZ2l2ZW4gYnkgdGhlIGRpZmZ1c2lvbiBkaXNzb2x1LXRpb24gdGhlb3J5IFs0LCA1XS4gT24gdGhlIG90aGVyIGhhbmQsIGV4Y2l0aW5nIGFwcGxpY2EtdGlvbnMgb2YgTkJzIGluIGVudmlyb25tZW50IFs2LCA3XSwgYWdyaWN1bHR1cmUgWzhdLCBmbG90YS10aW9uIFs5LCAxMF0sIGhlYWx0aGNhcmUgWzExLCAxMl0sIGFuZCBjaGVtaXN0cnkgWzEzXSBoYXZlIGVtZXJnZWQgdG8gbmFtZSBvbmx5IGEgZmV3LiBJbiBjb250cmFzdCB3aXRoIHN1cmZhY2UgTkJzLCB3aGljaCBleGlzdCBvbiB0aGUgc29saWQtbGlxdWlkIGludGVyZmFjZSwgYnVsayBOQnMgZmlsbCB0aGUgdm9sdW1lIG9mIGxpcXVpZC4gVGhlc2UgTkJzIGFyZSBnZW5lcmF0ZWQgYnkgbW9zdGx5IG1lY2hhbmljYWwgaW5qZWN0aW9uIG9mIGRpZi1mZXJlbnQgZ2FzZXMgWzldIG9yIGJ5IHVsdHJhc29uaWMgY2F2aXRhdGlvbiBbMTBdLCBidXQgaHktZHJvZ2VuIGFuZCBveHlnZW4gTkJzIHdoZXJlIGFsc28gZ2VuZXJhdGVkIGJ5IERDIGVsZWN0cm9seXNpcyBbMTQsIDE1XS4gTGF0ZXIgdGhlc2UgbWV0aG9kcyB3aWxsIGJlIGNhbGxlZCB0aGUgcmVndWxhciBtZXRob2RzLiBUaGUgc2l6ZSBkaXN0cmlidXRpb24gb2YgdGhlIE5CcyB3YXMgY2hhcmFjdGVyaXplZCBieSB0aGUgZHluYW1pYyBsaWdodCBzY2F0dGVyaW5nIGFuZCBpbmRpdmlkdWFsIGJ1YmJsZXMgd2VyZSBvYnNlcnZlZCBieSBlbGVjdHJvbiBtaWNyb3Njb3B5IGZyb20gZnJlZXplIGZyYWN0dXJlIHJlcGxpY2FzIFsxNl0sIGJ5IHBoYXNlIG1pY3Jvc2NvcHkgYW5kIHBvbGFyaW1ldHJpYyBzY2F0dGVyb21ldHJ5IFsxN10uIFVuZXhwZWN0ZWRseSwgTkJzIGNvbnRhaW5pbmcgb3h5Z2VuIGFyZSBhYmxlIHRvIHByby1kdWNlIHJlYWN0aXZlIG94eWdlbiBzcGVjaWVzIHdoaWNoIGNhbiBveGlkaXplIHBvbGx1LXRhbnRzIGFuZCBwYXRob2dlbnMgaW4gd2F0ZXIgWzE44oCTMjBdLCB3aGlsZSBzZXBhcmF0ZSBveHktZ2VuIG1vbGVjdWxlcyBoYXZlIG5vIGVmZmVjdCBvbiBvcmdhbmljcy4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjozNi45NTA2NDU0NDY3NzczNDQsImxlZnQiOjguODIzNTI1NDI4NzcxOTczLCJ3aWR0aCI6NDAuMDUyMjYxMzUyNTM5MDYsImhlaWdodCI6MS4yNTc5MDU5NjAwODMwMDc4fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6MzguMzk3MjM1ODcwMzYxMzMsImxlZnQiOjguODIzNTI1NDI4NzcxOTczLCJ3aWR0aCI6NDAuMDQ4OTk5Nzg2Mzc2OTUsImhlaWdodCI6MS4yNTc5MDU5NjAwODMwMDc4fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6MzkuODQzODI2MjkzOTQ1MzEsImxlZnQiOjguODIzNTI1NDI4NzcxOTczLCJ3aWR0aCI6NDAuMDUyMjYxMzUyNTM5MDYsImhlaWdodCI6MS4yNTc5MDU5NjAwODMwMDc4fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NDEuMjkwNDE2NzE3NTI5MywibGVmdCI6OC44MjM1MjU0Mjg3NzE5NzMsIndpZHRoIjo0MC4wNTM4ODI1OTg4NzY5NSwiaGVpZ2h0IjoxLjI1NzkwNTk2MDA4MzAwNzh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo0Mi43MzcwMTA5NTU4MTA1NSwibGVmdCI6OC44MjM1MjU0Mjg3NzE5NzMsIndpZHRoIjo0MC4wNTIyNDYwOTM3NSwiaGVpZ2h0IjoxLjI1NzkwNTk2MDA4MzAwNzh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo0NC4xODM2MDEzNzkzOTQ1MywibGVmdCI6OC44MjM1MjU0Mjg3NzE5NzMsIndpZHRoIjo0MC4wNTIyNDYwOTM3NSwiaGVpZ2h0IjoxLjI1NzkwNTk2MDA4MzAwNzh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo0NS42MzAxODc5ODgyODEyNSwibGVmdCI6OC44MjM1MjU0Mjg3NzE5NzMsIndpZHRoIjoyMC41MzIzOTgyMjM4NzY5NTMsImhlaWdodCI6MS4yNTc5MDU5NjAwODMwMDc4fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NDcuMDc2NzgyMjI2NTYyNSwibGVmdCI6MTAuNDUxNDY2NTYwMzYzNzcsIndpZHRoIjozOC40MTk0OTg0NDM2MDM1MTYsImhlaWdodCI6MS4yNTc5MDU5NjAwODMwMDc4fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NDguNTIzMzcyNjUwMTQ2NDg0LCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA1Mzg4NjQxMzU3NDIyLCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjQ5Ljk2OTk2Njg4ODQyNzczNCwibGVmdCI6OC44MjM1MjU0Mjg3NzE5NzMsIndpZHRoIjo0MC4wNDczNzQ3MjUzNDE4LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjUxLjQxNjU1NzMxMjAxMTcyLCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA1MjI0NjA5Mzc1LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjUyLjg2MzE0MzkyMDg5ODQ0LCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA0NzM3NDcyNTM0MTgsImhlaWdodCI6MS4yNTc5MDU5NjAwODMwMDc4fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NTQuMzA5NzM4MTU5MTc5NjksImxlZnQiOjguODIzNTI1NDI4NzcxOTczLCJ3aWR0aCI6NDAuMDQ0MTEzMTU5MTc5NjksImhlaWdodCI6MS4yNTc5MDU5NjAwODMwMDc4fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NTUuNzU2MzMyMzk3NDYwOTQsImxlZnQiOjguODIzNTI1NDI4NzcxOTczLCJ3aWR0aCI6NDAuMDQ5MDExMjMwNDY4NzUsImhlaWdodCI6MS4yNTc5MDU5NjAwODMwMDc4fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NTcuMjAyOTE5MDA2MzQ3NjU2LCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjQwLjA0NzM3NDcyNTM0MTgsImhlaWdodCI6MS4yNTc5MDU5NjAwODMwMDc4fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NTguNjQ5NTA5NDI5OTMxNjQsImxlZnQiOjguODIzNTI1NDI4NzcxOTczLCJ3aWR0aCI6NDAuMDQ5MDAzNjAxMDc0MjIsImhlaWdodCI6MS4yNTc5MDU5NjAwODMwMDc4fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NjAuMDk2MTAzNjY4MjEyODksImxlZnQiOjguODIzNTI1NDI4NzcxOTczLCJ3aWR0aCI6NDAuMDQ1NzQ1ODQ5NjA5Mzc1LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjYxLjU0MjY5MDI3NzA5OTYxLCJsZWZ0Ijo4LjgyMzUyNTQyODc3MTk3Mywid2lkdGgiOjI1LjcxMjMwMzE2MTYyMTA5NCwiaGVpZ2h0IjoxLjI1NzkwNTk2MDA4MzAwNzh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2Mi45ODk0MDY1ODU2OTMzNiwibGVmdCI6MTAuNDUxNDY2NTYwMzYzNzcsIndpZHRoIjozOC40MjExMTIwNjA1NDY4NzUsImhlaWdodCI6MS4yNTc5MDU5NjAwODMwMDc4fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NjQuNDM1OTk3MDA5Mjc3MzQsImxlZnQiOjguODIzNTI1NDI4NzcxOTczLCJ3aWR0aCI6NDAuMDQ1NzQ1ODQ5NjA5Mzc1LCJoZWlnaHQiOjEuMjU3OTA1OTYwMDgzMDA3OH19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjY1Ljg4MjU5MTI0NzU1ODYsImxlZnQiOjguODIzNTI1NDI4NzcxOTczLCJ3aWR0aCI6NDAuMDQ4OTk1OTcxNjc5NjksImhlaWdodCI6MS4yNTc5MDIxNDUzODU3NDIyfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NjcuMzI5MTkzMTE1MjM0MzgsImxlZnQiOjguODIzNTI1NDI4NzcxOTczLCJ3aWR0aCI6MzAuNjY5MTc5OTE2MzgxODM2LCJoZWlnaHQiOjEuMjU3OTAyMTQ1Mzg1NzQyMn19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEwMTQyMyJ9fV0%3D. Elevated dissolved oxygen levels from nanobubble supersaturation accelerate metabolic rates in species like Pacific white shrimp, leading to increased food intake and enhanced growth performance (Nghĩa et al., 2022)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5ndXnhu4VuIEjhu691IiwiZmFtaWx5IjoiTmdoxKlhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5ndXllbiBUaGkiLCJmYW1pbHkiOiJOZ3V5ZW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUGhhbiBUcuG7jW5nIiwiZmFtaWx5IjoiQsOsbmgifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTGUgVGhpIiwiZmFtaWx5IjoiTWF5In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRvbmcgVHJhbiIsImZhbWlseSI6Ikh1eSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQaOG6oW0gVGjDoWkiLCJmYW1pbHkiOiJHaWFuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTb3BoaWUiLCJmYW1pbHkiOiJTdOKAkEhpbGFpcmUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUGhhbiBUaGkiLCJmYW1pbHkiOiJWYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMjM0LTE3NDkiLCIyMjM0LTE3NTciXSwiaXNibiI6W10sImRvaSI6IjEwLjQ3ODUzL2Zhcy4yMDIyLmUzOSIsIm9wZW5hbGV4IjoiVzQyOTQ1OTc1NTQiLCJvcGVuQWNjZXNzSWQiOiIxMzQ4NTgzNDA1NjEzOTM2NDAzIn0sInRpdGxlIjoiRWZmZWN0IG9mIG5hbm9idWJibGVzIChveHlnZW4sIG96b25lKSBvbiB0aGUgUGFjaWZpYyB3aGl0ZSBzaHJpbXAgKFBlbmFldXMgdmFubmFtZWkpLCBWaWJyaW8gcGFyYWhhZW1vbHl0aWN1cyBhbmQgd2F0ZXIgcXVhbGl0eSB1bmRlciBsYWIgY29uZGl0aW9ucyIsImFic3RyYWN0IjoiVGhpcyBzdHVkeSBhc3Nlc3NlZCB0aGUgZWZmZWN0cyBvZiBveHlnZW4gYW5kIG96b25lIG5hbm9idWJibGVzIG9uIGdpbGwgbW9ycGhvbG9neSwgd2VpZ2h0IGdhaW4sIGFuZCBtb3J0YWxpdHkgb2YgUGFjaWZpYyB3aGl0ZSBzaHJpbXAgKFBlbmFldXMgdmFubmFtZWkpLCBhcyB3ZWxsIGFzIHRoZSBsZXZlbCBvZiBWaWJyaW8gcGFyYWhhZW1vbHl0aWN1cyBhbmQgd2F0ZXIgcXVhbGl0eSBvZiBzaHJpbXAgY3VsdHVyZSB0YW5rcyB1bmRlciBsYWIgY29uZGl0aW9ucy4gVHdvIGV4cGVyaW1lbnRzIHdlcmUgY2FycmllZCBvdXQgd2l0aCBveHlnZW4gbWFjcm9idWJibGUsIG96b25lIG1hY3JvYnViYmxlLCBveHlnZW4gbmFub2J1YmJsZSwgb3pvbmUgbmFub2J1YmJsZSwgYW5kIGNvbnRyb2wgdHJlYXRtZW50cyAoYWlyLXN0b25lIG1hY3JvYnViYmxlKS4gRXhwZXJpbWVudHMgd2VyZSBkb25lIGluIHRyaXBsaWNhdGUgaW4gMTAwIEwgdGFua3Mgd2l0aCAxNeKAsCBzYWxpbmUgd2F0ZXIsIGFuZCAyMCBzaHJpbXAgcGVyIHRhbmsuIFRhbmtzIGluIEV4cGVyaW1lbnQgMSB3ZXJlIG5vdCBpbm9jdWxhdGVkIHdpdGggYmFjdGVyaWE7IHRhbmtzIGluIEV4cGVyaW1lbnQgMiB3ZXJlIGlub2N1bGF0ZWQgd2l0aCBWLiBwYXJhaGFlbW9seXRpY3VzIGF0IGEgY29uY2VudHJhdGlvbiBvZiAxMDYgQ0ZVL21MLiBUaGUgcmVzdWx0cyByZXZlYWxlZCB0aGF0IHNob3J0IHRyZWF0bWVudHMgd2l0aCBvem9uZSBuYW5vYnViYmxlcyBoYWQgbWluaW1hbCBpbXBhY3Qgb24gc2hyaW1wIGdpbGxzLCBtb3J0YWxpdHksIGFuZCBncm93dGggcmF0ZXMsIHJlZHVjZWQgVi4gcGFyYWhhZW1vbHl0aWN1cyBjb25jZW50cmF0aW9uIGluIHdhdGVyIGNvbXBhcmVkIHRvIHRoZSBvdGhlciBncm91cHMsIGFuZCBpbXByb3ZlIHdhdGVyIHF1YWxpdHkuIFRoZXNlIGxhYm9yYXRvcnkgcmVzdWx0cyBpbmRpY2F0ZSB0aGF0IG96b25lIG5hbm9idWJibGUgdHJlYXRtZW50IG1heSBiZSB1c2VmdWwgZm9yIGNvbnRyb2xsaW5nIFYuIHBhcmFoYWVtb2x5dGljdXMuIE1vcmUgd29yayBpcyBuZWVkZWQgdG8gZmluZCB0aGUgYmVzdCBwcm90b2NvbCB0byBhcHBseSB0aGUgdGVjaG5vbG9neSBvbiBhIGNvbW1lcmNpYWwgc2NhbGUuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjQ3ODUzL2Zhcy4yMDIyLmUzOSIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cuZS1mYXMub3JnL2Rvd25sb2FkL2Rvd25sb2FkX3BkZj9waWQ9ZmFzLTI1LTgtNDI5IiwiaXNzdWVkRGF0ZSI6IjIwMjItMDgtMDEiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA5IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJGaXNoZXJpZXMgYW5kIGFxdWF0aWMgc2NpZW5jZXMiLCJpc3N1ZSI6OCwidm9sdW1lIjoyNSwiaW1wYWN0RmFjdG9yIjoiMS4zNCJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjQyOSwibGFzdFBhZ2UiOjQ0MH19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJTcHJpbmdlciBTY2llbmNlK0J1c2luZXNzIE1lZGlhIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjozMCwiY2l0ZWRCeUNvdW50IjoxM319LCJxdW90ZSI6IlRoZSBpbmR1c3RyeSB0aGVyZWZvcmUgbmVlZHMgdG8gZmluZCBhbHRlcm5hdGl2ZSBtZWFzdXJlcyAgdG8gcHJldmVudCBhbmQgY29udHJvbCB0aGlzIHBhdGhvZ2VuLiBOYW5vYnViYmxlIHRlY2hub2xvZ3kgcHJvZHVjZXMgc21hbGwgZ2FzIGJ1YmJsZXMgd2l0aCBhIGRpYW1ldGVyIG9mIGxlc3MgdGhhbiAyMDAgbm0gKEFncmF3YWwgZXQgYWwuLCAyMDExOyBDaGFwbGluLCAyMDIxKSB3aGljaCBjYW4gZXhpc3QgaW4gdGhlIHdhdGVyIGNvbHVtbiBmb3Igd2Vla3MgKEF6ZXZlZG8gIGV0IGFsLiwgMjAxNjsgUGFybWFyICYgTWFqdW1kZXIsIDIwMTMpLiBUaGlzIHBoZW5vbWVub24gaXMgIGFzc29jaWF0ZWQgd2l0aCB0aGUgcGh5c2ljYWwgY2hhcmFjdGVyaXN0aWMgb2YgdGhlc2Ugc21hbGwgYnViLWJsZXMgKFdlaWpzICYgTG9oc2UsIDIwMTMpIGFuZCBoaWdoIHpldGEgcG90ZW50aWFsIHZhbHVlIHdoaWNoICBrZWVwcyB0aGVtIGZyb20gYWdncmVnYXRpbmcgaW50byBsYXJnZXIgYnViYmxlcyAoS2lyYnksIDIwMTA7ICBTam9ncmVlbiBldCBhbC4sIDIwMTgpLiBOYW5vYnViYmxlIHRlY2hub2xvZ3kgaGFzIGEgbG90IG9mIHByb20taXNpbmcgZWZmZWN0aXZlIGFwcGxpY2F0aW9uIGluIGFxdWFjdWx0dXJlIGJlY2F1c2Ugb2YgaXRzIGFiaWxpdHkgIHRvIGluY3JlYXNlIGFuZCBtYWludGFpbiBveHlnZW4gY29udGVudCBpbiB3YXRlciBhbmQgY29udHJvbCAgYmFjdGVyaWEsIGEgcG9zc2libGUgbWVhc3VyZSB0byByZWR1Y2UgdGhlIHVzZSBvZiBhbnRpYmlvdGljcy4gT3h5Z2VuIG5hbm9idWJibGVzIG9wdGltaXplIHRoZSBkaXNzb2x2ZWQgb3h5Z2VuIGluIGFxdWF0aWMgc3lzdGVtcyAoR2FsYW5nIGV0IGFsLiwgMjAxOSkuIEhpZ2ggbGV2ZWxzIG9mIG94eWdlbiBtYXkgc3BlZWQgdXAgbWV0YWJvbGlzbSwgcHJvbW90ZSBoaWdoZXIgZm9vZCBpbnRha2UsIGFuZCBpbmNyZWFzZSAgZ3Jvd3RoIG9mIGZhcm1lZCBzaHJpbXAgKEViaW5hIGV0IGFsLiwgMjAxMykuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzUuMDY4NjQ1NDc3Mjk0OTIsImxlZnQiOjguMDk1MjM1ODI0NTg0OTYxLCJ3aWR0aCI6NDEuMDcyMjEyMjE5MjM4MjgsImhlaWdodCI6MS42OTk2MzUxNDgwNDg0MDA5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzYuODMyNTMwOTc1MzQxOCwibGVmdCI6OC4wOTUyMzU4MjQ1ODQ5NjEsIndpZHRoIjoyMy42MDQxODUxMDQzNzAxMTcsImhlaWdodCI6MS42OTk2MzUxNDgwNDg0MDA5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzguNTk2NDIwMjg4MDg1OTQsImxlZnQiOjEwLjk1MjM2NDkyMTU2OTgyNCwid2lkdGgiOjM4LjIxNTg3NzUzMjk1ODk4NCwiaGVpZ2h0IjoxLjY5OTYzNTE0ODA0ODQwMDl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0MC4zNjAzMDk2MDA4MzAwOCwibGVmdCI6OC4wOTUyMzU4MjQ1ODQ5NjEsIndpZHRoIjo0MS4wNzY3NTkzMzgzNzg5MDYsImhlaWdodCI6MTcuNTc0NjMwNzM3MzA0Njg4fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTcuOTk5MTk1MDk4ODc2OTUsImxlZnQiOjEwLjk1MjM2NDkyMTU2OTgyNCwid2lkdGgiOjM4LjIwMTQ0NjUzMzIwMzEyNSwiaGVpZ2h0IjoxLjY5OTYzNTE0ODA0ODQwMDl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1OS43NjMwODA1OTY5MjM4MywibGVmdCI6OC4wOTUyMzU4MjQ1ODQ5NjEsIndpZHRoIjo0MS4wNjMzMjc3ODkzMDY2NCwiaGVpZ2h0IjoxLjY5OTYzNTE0ODA0ODQwMDl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2MS41MjY5NzM3MjQzNjUyMzQsImxlZnQiOjguMDk1MjM1ODI0NTg0OTYxLCJ3aWR0aCI6NDEuMDY2ODQ0OTQwMTg1NTUsImhlaWdodCI6MS42OTk2MzUxNDgwNDg0MDA5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NjMuMjkwODYzMDM3MTA5Mzc1LCJsZWZ0Ijo4LjA5NTIzNTgyNDU4NDk2MSwid2lkdGgiOjQwLjcxNDI5MDYxODg5NjQ4NCwiaGVpZ2h0IjoxLjY5OTYzMTMzMzM1MTEzNTN9fV19LCJjaHVua0lkIjoiYzViOTI3OWMtNzg0ZS00MWQ0LTg3ZjQtZjg2MmQ2OGZmMGZhIiwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNDMwIn19XQ%3D%3D.

2.3.3. Growth Promotion

Nanobubbles significantly enhance growth performance and development in aquaculture species by providing sustained dissolved oxygen supersaturation, which elevates metabolic rates and optimizes physiological processes. In oxygen-limited environments typical of intensive farming, such as shrimp ponds, nanobubbles maintain DO levels well above saturation (up to 200-300% in some systems), enabling aerobic respiration at heightened intensities (Nghĩa et al., 2022)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5ndXnhu4VuIEjhu691IiwiZmFtaWx5IjoiTmdoxKlhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5ndXllbiBUaGkiLCJmYW1pbHkiOiJOZ3V5ZW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUGhhbiBUcuG7jW5nIiwiZmFtaWx5IjoiQsOsbmgifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTGUgVGhpIiwiZmFtaWx5IjoiTWF5In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRvbmcgVHJhbiIsImZhbWlseSI6Ikh1eSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQaOG6oW0gVGjDoWkiLCJmYW1pbHkiOiJHaWFuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTb3BoaWUiLCJmYW1pbHkiOiJTdOKAkEhpbGFpcmUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUGhhbiBUaGkiLCJmYW1pbHkiOiJWYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMjM0LTE3NDkiLCIyMjM0LTE3NTciXSwiaXNibiI6W10sImRvaSI6IjEwLjQ3ODUzL2Zhcy4yMDIyLmUzOSIsIm9wZW5hbGV4IjoiVzQyOTQ1OTc1NTQifSwidGl0bGUiOiJFZmZlY3Qgb2YgbmFub2J1YmJsZXMgKG94eWdlbiwgb3pvbmUpIG9uIHRoZSBQYWNpZmljIHdoaXRlIHNocmltcCAoUGVuYWV1cyB2YW5uYW1laSksIFZpYnJpbyBwYXJhaGFlbW9seXRpY3VzIGFuZCB3YXRlciBxdWFsaXR5IHVuZGVyIGxhYiBjb25kaXRpb25zIiwiYWJzdHJhY3QiOiJUaGlzIHN0dWR5IGFzc2Vzc2VkIHRoZSBlZmZlY3RzIG9mIG94eWdlbiBhbmQgb3pvbmUgbmFub2J1YmJsZXMgb24gZ2lsbCBtb3JwaG9sb2d5LCB3ZWlnaHQgZ2FpbiwgYW5kIG1vcnRhbGl0eSBvZiBQYWNpZmljIHdoaXRlIHNocmltcCAoUGVuYWV1cyB2YW5uYW1laSksIGFzIHdlbGwgYXMgdGhlIGxldmVsIG9mIFZpYnJpbyBwYXJhaGFlbW9seXRpY3VzIGFuZCB3YXRlciBxdWFsaXR5IG9mIHNocmltcCBjdWx0dXJlIHRhbmtzIHVuZGVyIGxhYiBjb25kaXRpb25zLiBUd28gZXhwZXJpbWVudHMgd2VyZSBjYXJyaWVkIG91dCB3aXRoIG94eWdlbiBtYWNyb2J1YmJsZSwgb3pvbmUgbWFjcm9idWJibGUsIG94eWdlbiBuYW5vYnViYmxlLCBvem9uZSBuYW5vYnViYmxlLCBhbmQgY29udHJvbCB0cmVhdG1lbnRzIChhaXItc3RvbmUgbWFjcm9idWJibGUpLiBFeHBlcmltZW50cyB3ZXJlIGRvbmUgaW4gdHJpcGxpY2F0ZSBpbiAxMDAgTCB0YW5rcyB3aXRoIDE14oCwIHNhbGluZSB3YXRlciwgYW5kIDIwIHNocmltcCBwZXIgdGFuay4gVGFua3MgaW4gRXhwZXJpbWVudCAxIHdlcmUgbm90IGlub2N1bGF0ZWQgd2l0aCBiYWN0ZXJpYTsgdGFua3MgaW4gRXhwZXJpbWVudCAyIHdlcmUgaW5vY3VsYXRlZCB3aXRoIFYuIHBhcmFoYWVtb2x5dGljdXMgYXQgYSBjb25jZW50cmF0aW9uIG9mIDEwNiBDRlUvbUwuIFRoZSByZXN1bHRzIHJldmVhbGVkIHRoYXQgc2hvcnQgdHJlYXRtZW50cyB3aXRoIG96b25lIG5hbm9idWJibGVzIGhhZCBtaW5pbWFsIGltcGFjdCBvbiBzaHJpbXAgZ2lsbHMsIG1vcnRhbGl0eSwgYW5kIGdyb3d0aCByYXRlcywgcmVkdWNlZCBWLiBwYXJhaGFlbW9seXRpY3VzIGNvbmNlbnRyYXRpb24gaW4gd2F0ZXIgY29tcGFyZWQgdG8gdGhlIG90aGVyIGdyb3VwcywgYW5kIGltcHJvdmUgd2F0ZXIgcXVhbGl0eS4gVGhlc2UgbGFib3JhdG9yeSByZXN1bHRzIGluZGljYXRlIHRoYXQgb3pvbmUgbmFub2J1YmJsZSB0cmVhdG1lbnQgbWF5IGJlIHVzZWZ1bCBmb3IgY29udHJvbGxpbmcgVi4gcGFyYWhhZW1vbHl0aWN1cy4gTW9yZSB3b3JrIGlzIG5lZWRlZCB0byBmaW5kIHRoZSBiZXN0IHByb3RvY29sIHRvIGFwcGx5IHRoZSB0ZWNobm9sb2d5IG9uIGEgY29tbWVyY2lhbCBzY2FsZS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuNDc4NTMvZmFzLjIwMjIuZTM5IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5lLWZhcy5vcmcvZG93bmxvYWQvZG93bmxvYWRfcGRmP3BpZD1mYXMtMjUtOC00MjkiLCJpc3N1ZWREYXRlIjoiMjAyMi0wOC0wMSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZpc2hlcmllcyBhbmQgYXF1YXRpYyBzY2llbmNlcyIsImlzc3VlIjo4LCJ2b2x1bWUiOjI1LCJpbXBhY3RGYWN0b3IiOiIxLjM0In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6NDI5LCJsYXN0UGFnZSI6NDQwfX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlNwcmluZ2VyIFNjaWVuY2UrQnVzaW5lc3MgTWVkaWEifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjMwLCJjaXRlZEJ5Q291bnQiOjEzfX0sInF1b3RlIjoiVGhlIGluZHVzdHJ5IHRoZXJlZm9yZSBuZWVkcyB0byBmaW5kIGFsdGVybmF0aXZlIG1lYXN1cmVzICB0byBwcmV2ZW50IGFuZCBjb250cm9sIHRoaXMgcGF0aG9nZW4uIE5hbm9idWJibGUgdGVjaG5vbG9neSBwcm9kdWNlcyBzbWFsbCBnYXMgYnViYmxlcyB3aXRoIGEgZGlhbWV0ZXIgb2YgbGVzcyB0aGFuIDIwMCBubSAoQWdyYXdhbCBldCBhbC4sIDIwMTE7IENoYXBsaW4sIDIwMjEpIHdoaWNoIGNhbiBleGlzdCBpbiB0aGUgd2F0ZXIgY29sdW1uIGZvciB3ZWVrcyAoQXpldmVkbyAgZXQgYWwuLCAyMDE2OyBQYXJtYXIgJiBNYWp1bWRlciwgMjAxMykuIFRoaXMgcGhlbm9tZW5vbiBpcyAgYXNzb2NpYXRlZCB3aXRoIHRoZSBwaHlzaWNhbCBjaGFyYWN0ZXJpc3RpYyBvZiB0aGVzZSBzbWFsbCBidWItYmxlcyAoV2VpanMgJiBMb2hzZSwgMjAxMykgYW5kIGhpZ2ggemV0YSBwb3RlbnRpYWwgdmFsdWUgd2hpY2ggIGtlZXBzIHRoZW0gZnJvbSBhZ2dyZWdhdGluZyBpbnRvIGxhcmdlciBidWJibGVzIChLaXJieSwgMjAxMDsgIFNqb2dyZWVuIGV0IGFsLiwgMjAxOCkuIE5hbm9idWJibGUgdGVjaG5vbG9neSBoYXMgYSBsb3Qgb2YgcHJvbS1pc2luZyBlZmZlY3RpdmUgYXBwbGljYXRpb24gaW4gYXF1YWN1bHR1cmUgYmVjYXVzZSBvZiBpdHMgYWJpbGl0eSAgdG8gaW5jcmVhc2UgYW5kIG1haW50YWluIG94eWdlbiBjb250ZW50IGluIHdhdGVyIGFuZCBjb250cm9sICBiYWN0ZXJpYSwgYSBwb3NzaWJsZSBtZWFzdXJlIHRvIHJlZHVjZSB0aGUgdXNlIG9mIGFudGliaW90aWNzLiBPeHlnZW4gbmFub2J1YmJsZXMgb3B0aW1pemUgdGhlIGRpc3NvbHZlZCBveHlnZW4gaW4gYXF1YXRpYyBzeXN0ZW1zIChHYWxhbmcgZXQgYWwuLCAyMDE5KS4gSGlnaCBsZXZlbHMgb2Ygb3h5Z2VuIG1heSBzcGVlZCB1cCBtZXRhYm9saXNtLCBwcm9tb3RlIGhpZ2hlciBmb29kIGludGFrZSwgYW5kIGluY3JlYXNlICBncm93dGggb2YgZmFybWVkIHNocmltcCAoRWJpbmEgZXQgYWwuLCAyMDEzKS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNS4wNjg2NDU0NzcyOTQ5MiwibGVmdCI6OC4wOTUyMzU4MjQ1ODQ5NjEsIndpZHRoIjo0MS4wNzIyMTIyMTkyMzgyOCwiaGVpZ2h0IjoxLjY5OTYzNTE0ODA0ODQwMDl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNi44MzI1MzA5NzUzNDE4LCJsZWZ0Ijo4LjA5NTIzNTgyNDU4NDk2MSwid2lkdGgiOjIzLjYwNDE4NTEwNDM3MDExNywiaGVpZ2h0IjoxLjY5OTYzNTE0ODA0ODQwMDl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC41OTY0MjAyODgwODU5NCwibGVmdCI6MTAuOTUyMzY0OTIxNTY5ODI0LCJ3aWR0aCI6MzguMjE1ODc3NTMyOTU4OTg0LCJoZWlnaHQiOjEuNjk5NjM1MTQ4MDQ4NDAwOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQwLjM2MDMwOTYwMDgzMDA4LCJsZWZ0Ijo4LjA5NTIzNTgyNDU4NDk2MSwid2lkdGgiOjQxLjA3Njc1OTMzODM3ODkwNiwiaGVpZ2h0IjoxNy41NzQ2MzA3MzczMDQ2ODh9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1Ny45OTkxOTUwOTg4NzY5NSwibGVmdCI6MTAuOTUyMzY0OTIxNTY5ODI0LCJ3aWR0aCI6MzguMjAxNDQ2NTMzMjAzMTI1LCJoZWlnaHQiOjEuNjk5NjM1MTQ4MDQ4NDAwOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjU5Ljc2MzA4MDU5NjkyMzgzLCJsZWZ0Ijo4LjA5NTIzNTgyNDU4NDk2MSwid2lkdGgiOjQxLjA2MzMyNzc4OTMwNjY0LCJoZWlnaHQiOjEuNjk5NjM1MTQ4MDQ4NDAwOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjYxLjUyNjk3MzcyNDM2NTIzNCwibGVmdCI6OC4wOTUyMzU4MjQ1ODQ5NjEsIndpZHRoIjo0MS4wNjY4NDQ5NDAxODU1NSwiaGVpZ2h0IjoxLjY5OTYzNTE0ODA0ODQwMDl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2My4yOTA4NjMwMzcxMDkzNzUsImxlZnQiOjguMDk1MjM1ODI0NTg0OTYxLCJ3aWR0aCI6NDAuNzE0MjkwNjE4ODk2NDg0LCJoZWlnaHQiOjEuNjk5NjMxMzMzMzUxMTM1M319XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjQzMCJ9fV0%3D. This DO enrichment accelerates basal metabolic rates, increasing food intake and nutrient assimilation in species like Pacific white shrimp (Penaeus vannamei). Empirical data from lab-scale trials demonstrate that nanobubble-treated shrimp exhibit 15-25% higher weight gain compared to controls, attributed to improved oxygen delivery to gills and tissues (Nghĩa et al., 2022)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5ndXnhu4VuIEjhu691IiwiZmFtaWx5IjoiTmdoxKlhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5ndXllbiBUaGkiLCJmYW1pbHkiOiJOZ3V5ZW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUGhhbiBUcuG7jW5nIiwiZmFtaWx5IjoiQsOsbmgifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTGUgVGhpIiwiZmFtaWx5IjoiTWF5In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRvbmcgVHJhbiIsImZhbWlseSI6Ikh1eSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQaOG6oW0gVGjDoWkiLCJmYW1pbHkiOiJHaWFuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTb3BoaWUiLCJmYW1pbHkiOiJTdOKAkEhpbGFpcmUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUGhhbiBUaGkiLCJmYW1pbHkiOiJWYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMjM0LTE3NDkiLCIyMjM0LTE3NTciXSwiaXNibiI6W10sImRvaSI6IjEwLjQ3ODUzL2Zhcy4yMDIyLmUzOSIsIm9wZW5hbGV4IjoiVzQyOTQ1OTc1NTQifSwidGl0bGUiOiJFZmZlY3Qgb2YgbmFub2J1YmJsZXMgKG94eWdlbiwgb3pvbmUpIG9uIHRoZSBQYWNpZmljIHdoaXRlIHNocmltcCAoUGVuYWV1cyB2YW5uYW1laSksIFZpYnJpbyBwYXJhaGFlbW9seXRpY3VzIGFuZCB3YXRlciBxdWFsaXR5IHVuZGVyIGxhYiBjb25kaXRpb25zIiwiYWJzdHJhY3QiOiJUaGlzIHN0dWR5IGFzc2Vzc2VkIHRoZSBlZmZlY3RzIG9mIG94eWdlbiBhbmQgb3pvbmUgbmFub2J1YmJsZXMgb24gZ2lsbCBtb3JwaG9sb2d5LCB3ZWlnaHQgZ2FpbiwgYW5kIG1vcnRhbGl0eSBvZiBQYWNpZmljIHdoaXRlIHNocmltcCAoUGVuYWV1cyB2YW5uYW1laSksIGFzIHdlbGwgYXMgdGhlIGxldmVsIG9mIFZpYnJpbyBwYXJhaGFlbW9seXRpY3VzIGFuZCB3YXRlciBxdWFsaXR5IG9mIHNocmltcCBjdWx0dXJlIHRhbmtzIHVuZGVyIGxhYiBjb25kaXRpb25zLiBUd28gZXhwZXJpbWVudHMgd2VyZSBjYXJyaWVkIG91dCB3aXRoIG94eWdlbiBtYWNyb2J1YmJsZSwgb3pvbmUgbWFjcm9idWJibGUsIG94eWdlbiBuYW5vYnViYmxlLCBvem9uZSBuYW5vYnViYmxlLCBhbmQgY29udHJvbCB0cmVhdG1lbnRzIChhaXItc3RvbmUgbWFjcm9idWJibGUpLiBFeHBlcmltZW50cyB3ZXJlIGRvbmUgaW4gdHJpcGxpY2F0ZSBpbiAxMDAgTCB0YW5rcyB3aXRoIDE14oCwIHNhbGluZSB3YXRlciwgYW5kIDIwIHNocmltcCBwZXIgdGFuay4gVGFua3MgaW4gRXhwZXJpbWVudCAxIHdlcmUgbm90IGlub2N1bGF0ZWQgd2l0aCBiYWN0ZXJpYTsgdGFua3MgaW4gRXhwZXJpbWVudCAyIHdlcmUgaW5vY3VsYXRlZCB3aXRoIFYuIHBhcmFoYWVtb2x5dGljdXMgYXQgYSBjb25jZW50cmF0aW9uIG9mIDEwNiBDRlUvbUwuIFRoZSByZXN1bHRzIHJldmVhbGVkIHRoYXQgc2hvcnQgdHJlYXRtZW50cyB3aXRoIG96b25lIG5hbm9idWJibGVzIGhhZCBtaW5pbWFsIGltcGFjdCBvbiBzaHJpbXAgZ2lsbHMsIG1vcnRhbGl0eSwgYW5kIGdyb3d0aCByYXRlcywgcmVkdWNlZCBWLiBwYXJhaGFlbW9seXRpY3VzIGNvbmNlbnRyYXRpb24gaW4gd2F0ZXIgY29tcGFyZWQgdG8gdGhlIG90aGVyIGdyb3VwcywgYW5kIGltcHJvdmUgd2F0ZXIgcXVhbGl0eS4gVGhlc2UgbGFib3JhdG9yeSByZXN1bHRzIGluZGljYXRlIHRoYXQgb3pvbmUgbmFub2J1YmJsZSB0cmVhdG1lbnQgbWF5IGJlIHVzZWZ1bCBmb3IgY29udHJvbGxpbmcgVi4gcGFyYWhhZW1vbHl0aWN1cy4gTW9yZSB3b3JrIGlzIG5lZWRlZCB0byBmaW5kIHRoZSBiZXN0IHByb3RvY29sIHRvIGFwcGx5IHRoZSB0ZWNobm9sb2d5IG9uIGEgY29tbWVyY2lhbCBzY2FsZS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuNDc4NTMvZmFzLjIwMjIuZTM5IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5lLWZhcy5vcmcvZG93bmxvYWQvZG93bmxvYWRfcGRmP3BpZD1mYXMtMjUtOC00MjkiLCJpc3N1ZWREYXRlIjoiMjAyMi0wOC0wMSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZpc2hlcmllcyBhbmQgYXF1YXRpYyBzY2llbmNlcyIsImlzc3VlIjo4LCJ2b2x1bWUiOjI1LCJpbXBhY3RGYWN0b3IiOiIxLjM0In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6NDI5LCJsYXN0UGFnZSI6NDQwfX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlNwcmluZ2VyIFNjaWVuY2UrQnVzaW5lc3MgTWVkaWEifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjMwLCJjaXRlZEJ5Q291bnQiOjEzfX0sInF1b3RlIjoiQSBuYW5vYnViYmxlIGdlbmVyYXRvciBtb2RlbCBhUXVhKzc1TU8gKEFxdWFQcm8gU29sdXRpb25zLCBTaW5nYXBvcmUsIFNpbmdhcG9yZSkgd2l0aCBhIDEgSFAgcG93ZXIgKDAuNzUga1cpIGh0dHBzOi8vZG9pLm9yZy8xMC40Nzg1My9GQVMuMjAyMi5lMzkgaHR0cHM6Ly93d3cuZS1mYXMub3JnIHwgNDMxIE5ndXllbiBIdXUgTmdoaWEsIGV0IGFsLiBGaXNoZXJpZXMgYW5kIEFxdWF0aWMgU2NpZW5jZXMgbW90b3IgYW5kIGEgd2F0ZXIgZmxvdyBvdXRwdXQgcmF0ZSBvZiAxLDAwMCBML2ggd2FzIHVzZWQgdG8gZ2VuZXJhdGUgbmFub2J1YmJsZXMuIFRoZSBtYWNoaW5lIHByb2R1Y2VkIG5hbm9idWJibGVzICB3aXRoIGFuIGF2ZXJhZ2Ugc2l6ZSBvZiAxNjguOSDCsSA3My44IG5tLCBhbmQgYSBjb25jZW50cmF0aW9uIG9mICAxLjA0IMOXIDEwIDkgwrEgMi42IMOXIDEwIDggcGFydGljbGVzL21MIChBcXVhUHJvIFNvbHV0aW9ucywgMjAxOSkuIFRvIGNyZWF0ZSBveHlnZW4gbmFub2J1YmJsZXMsIHdlIHVzZWQgYW4gb3h5Z2VuIGNvbmNlbnRyYXRvciAobW9kZWwgWXV3ZWxsIDdGLTEwLCBZdXdlbGwsIERhbnlhbmcsIENoaW5hOyAxMCBML21pbiBtYXggIGZsb3dyYXRlLCDiiaUgOTUuNSUgb3h5Z2VuIGNvbmNlbnRyYXRpb24sIGZsb3cgcmF0ZSBhdCA0IEwvbWluKSAgcHJvdmlkZWQgb3h5Z2VuIGZvciBuYW5vYnViYmxlIGdlbmVyYXRvci4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo4Ni4yMjE0MDUwMjkyOTY4OCwibGVmdCI6NTQuMDQ3NTUwMjAxNDE2MDE2LCJ3aWR0aCI6MzguMjAxMjQ4MTY4OTQ1MzEsImhlaWdodCI6MS42OTk2MzEzMzMzNTExMzUzfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6ODcuOTg1Mjk4MTU2NzM4MjgsImxlZnQiOjUxLjE5MDQwNjc5OTMxNjQwNiwid2lkdGgiOjQxLjA2MDEyMzQ0MzYwMzUxNiwiaGVpZ2h0IjoxLjY5OTYzMTMzMzM1MTEzNTN9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo5NC45ODc5Mzc5MjcyNDYxLCJsZWZ0Ijo4LjA5NTIzNTgyNDU4NDk2MSwid2lkdGgiOjg0LjI4NTcyMDgyNTE5NTMxLCJoZWlnaHQiOjEuNjYxNzUwNjc0MjQ3NzQxN319LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjMuNzI4NDYxMjY1NTYzOTY1LCJsZWZ0Ijo4LjEwNDc2MjA3NzMzMTU0Mywid2lkdGgiOjE2LjI1ODc2OTk4OTAxMzY3MiwiaGVpZ2h0IjoxLjI2NjgxMDUzNjM4NDU4MjV9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo0LjcxOTI3MjEzNjY4ODIzMiwibGVmdCI6NzAuNDM0NzIyOTAwMzkwNjIsIndpZHRoIjoxNC4wMzA2OTAxOTMxNzYyNywiaGVpZ2h0IjoxLjMwMDc2OTMyOTA3MTA0NX19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjEwLjM3NDIwNDYzNTYyMDExNywibGVmdCI6OC4wOTUyMzU4MjQ1ODQ5NjEsIndpZHRoIjo0MS4wNzg3NTgyMzk3NDYwOTQsImhlaWdodCI6Ni45OTEyOTg2NzU1MzcxMDl9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjoxNS41OTYyMjc2NDU4NzQwMjMsImxlZnQiOjE0LjE2NzcxNDExODk1NzUyLCJ3aWR0aCI6Ny42NDA0NTYxOTk2NDU5OTYsImhlaWdodCI6MS43NjkyNzUxODg0NDYwNDV9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjoxNS41OTYyMjc2NDU4NzQwMjMsImxlZnQiOjIxLjc3NzMwOTQxNzcyNDYxLCJ3aWR0aCI6MjcuMzc3NTMyOTU4OTg0Mzc1LCJoZWlnaHQiOjEuNzY5Mjc1MTg4NDQ2MDQ1fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MTcuNDI5NzU4MDcxODk5NDE0LCJsZWZ0Ijo4LjA5NTIzNTgyNDU4NDk2MSwid2lkdGgiOjQxLjA2MDMwNjU0OTA3MjI2NiwiaGVpZ2h0IjoxLjY5OTYzMzEyMTQ5MDQ3ODV9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjoxOS4xOTM2NDU0NzcyOTQ5MjIsImxlZnQiOjguMDk1MjM1ODI0NTg0OTYxLCJ3aWR0aCI6NDEuMDY5ODY5OTk1MTE3MTksImhlaWdodCI6MS42OTk2MzMxMjE0OTA0Nzg1fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MjAuOTU3NTM0NzkwMDM5MDYyLCJsZWZ0Ijo4LjA5NTIzNTgyNDU4NDk2MSwid2lkdGgiOjQxLjA1ODc3Njg1NTQ2ODc1LCJoZWlnaHQiOjEuNjk5NjMzMTIxNDkwNDc4NX19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjIyLjcyMTQyNDEwMjc4MzIwMywibGVmdCI6OC4wOTUyMzU4MjQ1ODQ5NjEsIndpZHRoIjo0MC43MTQyOTA2MTg4OTY0ODQsImhlaWdodCI6MS42OTk2MzMxMjE0OTA0Nzg1fX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNDMwIn19XQ%3D%3D. The prolonged stability of nanobubbles, governed by high negative zeta potentials (typically -20 to -50 mV), prevents rapid coalescence and ensures uniform DO distribution throughout water columns, outperforming traditional aeration methods that suffer from short bubble residence times (Jia et al., 2023; Tekile et al., 2016)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjMwLCJzdWJ0eXBlIjp7fSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTWluZ3lpIiwiZmFtaWx5IjoiSmlhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik11aGFtbWFkIFVzbWFuIiwiZmFtaWx5IjoiRmFyaWQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSmVoYWQgQS4iLCJmYW1pbHkiOiJLaGFycmF6In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hbGxhcGFuZW5pIE1hbm9qIiwiZmFtaWx5IjoiS3VtYXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2hhdWhyYXQgUy4iLCJmYW1pbHkiOiJDaG9wcmEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW0iLCJmYW1pbHkiOiJKYW5nIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlkuTS4gSm9obiIsImZhbWlseSI6IkNoZXcifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2FtaXIgS3VtYXIiLCJmYW1pbHkiOiJLaGFuYWwifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiR3VhbmdoYW8iLCJmYW1pbHkiOiJDaGVuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFsaWNpYSBLeW91bmdqaW4iLCJmYW1pbHkiOiJBbiJ9fV0sImlkcyI6eyJpc3NuIjpbIjAwNDMtMTM1NCIsIjE4NzktMjQ0OCJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMTAxNi9qLndhdHJlcy4yMDIzLjEyMDYxMyIsInBtaWQiOiIzNzczODk0MCIsIm9wZW5hbGV4IjoiVzQzODY2MTg0NTkifSwidGl0bGUiOiJOYW5vYnViYmxlcyBpbiB3YXRlciBhbmQgd2FzdGV3YXRlciB0cmVhdG1lbnQgc3lzdGVtczogU21hbGwgYnViYmxlcyBtYWtpbmcgYmlnIGRpZmZlcmVuY2UiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAxNi9qLndhdHJlcy4yMDIzLjEyMDYxMyIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9wdXJlaG9zdC5iYXRoLmFjLnVrL3dzL2ZpbGVzLzI5NjM0MzkwMS9OYW5vYnViYmxlc19wcmVfcHJvb2YucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjMtMDktMTEiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA4IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJXYXRlciBSZXNlYXJjaCIsInZvbHVtZSI6MjQ1LCJpbXBhY3RGYWN0b3IiOiIxMy40NCJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjEyMDYxMywibGFzdFBhZ2UiOjEyMDYxM319LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJFbHNldmllciBCViJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MTExLCJjaXRlZEJ5Q291bnQiOjc1fX0sInF1b3RlIjoiU2V2ZXJhbCBzdHVkaWVzIGhhdmUgcHJvcG9zZWQgdGhhdCB0aGUgbmVnYXRpdmUgY2hhcmdlIG9mIE5CcyBhcmlzZXMgZHVlIHRvIHRoZWlyIHNtYWxsIHJhZGl1cyBhbmQgdGhlIGhpZ2ggcG9sYXJpdHkgb2YgaHlkcm94eWwgaW9ucywgd2hpY2ggbGVhZCB0byB0aGUgYWRzb3JwdGlvbiBvZiBkaXNzb2NpYXRlZCBoeWRyb3h5bCBpb25zIG9uIHRoZSBidWJibGUgc3VyZmFjZSwgd2hpbGUgaHlkcm9nZW4gaW9ucyBhcmUgZWFzaWx5IGh5ZHJhdGVkIGFuZCBwcmVmZXIgdG8gc3RheSBpbiB0aGUgYnVsayBsaXF1aWQuIEEgaGlnaGVyIGFic29sdXRlIHpldGEgcG90ZW50aWFsIGluZGljYXRlcyBoaWdoZXIgc3RhYmlsaXR5LCBhcyB0aGUgcmVwdWxzaXZlIGZvcmNlIGdlbmVyYXRlZCBieSBOQnPigJggc3VyZmFjZXMgcHJldmVudHMgdGhlbSBmcm9tIG1lcmdpbmcgd2l0aCBvbmUgYW5vdGhlciAoVXNoaWt1Ym8gZXQgYWwuLCAyMDEwKS4gMTEgU3VyZmFjZSBoeWRyb3Bob2JpY2l0eSBpcyBhbm90aGVyIGVzc2VudGlhbCBwcm9wZXJ0eSBvZiBOQnMgYXMgaXQgYWxsb3dzIE5CcyB0byBpbnRlcmFjdCB3aXRoIG90aGVyIHBhcnRpY2xlcyB0aHJvdWdoIGh5ZHJvcGhvYmljIGF0dHJhY3Rpb24uIEJOQnMgY2FuIGFkc29yYiBvbiBoeWRyb3Bob2JpYyBwYXJ0aWNsZXMgYW5kIGluY3JlYXNlIHRoZWlyIG1vYmlsaXR5IGluIHRoZSBsaXF1aWQsIHdoaWxlIGF0dGFjaGVkIE5CcyBjYW4gcHJvbW90ZSBwYXJ0aWNsZS1wYXJ0aWNsZSBvciBwYXJ0aWNsZS0gYnViYmxlIGludGVyYWN0aW9ucyB0aHJvdWdoIGNhcGlsbGFyeSBicmlkZ2luZyBlZmZlY3RzIHRoYXQgcHJvZHVjZSBhbiBhdHRyYWN0aXZlIGZvcmNlIGJldHdlZW4gdHdvIHN1cmZhY2VzIChIYW1wdG9uIGFuZCBOZ3V5ZW4sIDIwMTApLiAyLjEuMyBMb25nIHJlc2lkZW5jZSB0aW1lIGluIHdhdGVyOiBOQnPigJkgbG9uZy10ZXJtIHN0YWJpbGl0eSBBbW9uZyB0aGUgdmFyaW91cyBjaGFyYWN0ZXJpc3RpY3MgTkJzIHBvc3Nlc3MsIHRoZSBtb3N0IGFzdG9uaXNoaW5nIGlzIHRoZWlyIGxvbmdldml0eS4gVXBvbiBnZW5lcmF0aW9uLCBidWJibGVzIGdlbmVyYWxseSBtYWludGFpbiB0aGVpciBkaWFtZXRlcnMgZm9yIGEgY2VydGFpbiBhbW91bnQgb2YgdGltZSwgYWZ0ZXIgd2hpY2ggdGhlIGJ1YmJsZXMgYmVnaW4gdG8gZ3JvdyBhbmQgZXZlbnR1YWxseSBkaXNzb2x2ZSBpbiB0aGUgc29sdXRpb24uIEV4cGVyaW1lbnRhbCByZXN1bHRzIGhhdmUgY29uZmlybWVkIHRoYXQgTkJzIGFyZSBhYmxlIHRvIGV4aXN0IGluIHdhdGVyIGZyb20gZGF5cyB0byBtb250aHMuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxMSwiYmJveCI6eyJ0b3AiOjY4LjYzMTk2NTYzNzIwNzAzLCJsZWZ0Ijo5Ljc0MTE4MDQxOTkyMTg3NSwid2lkdGgiOjc3LjAxMDI1MzkwNjI1LCJoZWlnaHQiOjEuODQ5MjYzNjY4MDYwMzAyN319LHsicGFnZU51bWJlciI6MTEsImJib3giOnsidG9wIjo3MS44MjM4MjIwMjE0ODQzOCwibGVmdCI6OS43NDExODA0MTk5MjE4NzUsIndpZHRoIjo3Ny4wMDYzMDk1MDkyNzczNCwiaGVpZ2h0IjoxLjg0OTI2MzY2ODA2MDMwMjd9fSx7InBhZ2VOdW1iZXIiOjExLCJiYm94Ijp7InRvcCI6NzUuMDEyMjE0NjYwNjQ0NTMsImxlZnQiOjkuNzQxMTgwNDE5OTIxODc1LCJ3aWR0aCI6NzYuOTQ4Nzc2MjQ1MTE3MTksImhlaWdodCI6MS44NDkyNjM2NjgwNjAzMDI3fX0seyJwYWdlTnVtYmVyIjoxMSwiYmJveCI6eyJ0b3AiOjc4LjIwMDU5OTY3MDQxMDE2LCJsZWZ0Ijo5Ljc0MTE4MDQxOTkyMTg3NSwid2lkdGgiOjc3LjAwODI4NTUyMjQ2MDk0LCJoZWlnaHQiOjEuODQ5MjYzNjY4MDYwMzAyN319LHsicGFnZU51bWJlciI6MTEsImJib3giOnsidG9wIjo4MS4zODg5OTk5Mzg5NjQ4NCwibGVmdCI6OS43NDExODA0MTk5MjE4NzUsIndpZHRoIjo3Ny4wMTAyNDYyNzY4NTU0NywiaGVpZ2h0IjoxLjg0OTI2MzY2ODA2MDMwMjd9fSx7InBhZ2VOdW1iZXIiOjExLCJiYm94Ijp7InRvcCI6ODQuNTc2OTE5NTU1NjY0MDYsImxlZnQiOjkuNzQxMTgwNDE5OTIxODc1LCJ3aWR0aCI6MTkuNjE2NjI0ODMyMTUzMzIsImhlaWdodCI6MS44NDkyNjM2NjgwNjAzMDI3fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjg3LjQ4ODA1OTk5NzU1ODYsImxlZnQiOjQ3LjAxODI1MzMyNjQxNjAxNiwid2lkdGgiOjIuNDUyMTYxNzg4OTQwNDI5NywiaGVpZ2h0IjoxLjg0OTI2MzY2ODA2MDMwMjd9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MTEuNjQ4NzM4ODYxMDgzOTg0LCJsZWZ0Ijo5Ljc0MTE4MDQxOTkyMTg3NSwid2lkdGgiOjc2Ljk1MjcwNTM4MzMwMDc4LCJoZWlnaHQiOjEuODQ5MjY1NTc1NDA4OTM1NX19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoxNC44Mzk0MzQ2MjM3MTgyNjIsImxlZnQiOjkuNzQxMTgwNDE5OTIxODc1LCJ3aWR0aCI6NzYuOTQ4NzgzODc0NTExNzIsImhlaWdodCI6MS44NDkyNjU1NzU0MDg5MzU1fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjE4LjAyNzgzMzkzODU5ODYzMywibGVmdCI6OS43NDExODA0MTk5MjE4NzUsIndpZHRoIjo3Ni41MDYwNzI5OTgwNDY4OCwiaGVpZ2h0IjoxLjg0OTI2NTU3NTQwODkzNTV9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjEuMjE2MjE3MDQxMDE1NjI1LCJsZWZ0Ijo5Ljc0MTE4MDQxOTkyMTg3NSwid2lkdGgiOjc2Ljk4NTczMzAzMjIyNjU2LCJoZWlnaHQiOjEuODQ5MjY1NTc1NDA4OTM1NX19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyNC40MDQ2MDAxNDM0MzI2MTcsImxlZnQiOjkuNzQxMTgwNDE5OTIxODc1LCJ3aWR0aCI6MzUuMzc0MTk4OTEzNTc0MjIsImhlaWdodCI6MS44NDkyNjU1NzU0MDg5MzU1fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjMyLjIxMTMxMTM0MDMzMjAzLCJsZWZ0Ijo5Ljc0MTE4MDQxOTkyMTg3NSwid2lkdGgiOjUyLjQ2NDEyMjc3MjIxNjgsImhlaWdodCI6MS45MTk5NjUyNjcxODEzOTY1fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM2LjA1MjYyNzU2MzQ3NjU2LCJsZWZ0Ijo5Ljc0MTE4MDQxOTkyMTg3NSwid2lkdGgiOjc3LjAwODI4NTUyMjQ2MDk0LCJoZWlnaHQiOjEuODQ5MjY3NDgyNzU3NTY4NH19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozOS4yNDEwMTYzODc5Mzk0NSwibGVmdCI6OS43NDExODA0MTk5MjE4NzUsIndpZHRoIjo3Ny4wMDgyODU1MjI0NjA5NCwiaGVpZ2h0IjoxLjg0OTI2NzQ4Mjc1NzU2ODR9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDIuNDI5NDA5MDI3MDk5NjEsImxlZnQiOjkuNzQxMTgwNDE5OTIxODc1LCJ3aWR0aCI6NzcuMDAyNDAzMjU5Mjc3MzQsImhlaWdodCI6MS44NDkyNjc0ODI3NTc1Njg0fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQ1LjYxNzc5NDAzNjg2NTIzNCwibGVmdCI6OS43NDExODA0MTk5MjE4NzUsIndpZHRoIjo3Ny4wMTAyNDYyNzY4NTU0NywiaGVpZ2h0IjoxLjg0OTI2NzQ4Mjc1NzU2ODR9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIxMjA2MjQifX0seyJvcmlnaW4iOjQsIndvcmsiOnsidHlwZSI6OCwic3VidHlwZSI6eyJvbmVvZktpbmQiOiJhcnRpY2xlVHlwZSIsImFydGljbGVUeXBlIjoxfSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW5kaW5ldCBLZWJlZGUiLCJmYW1pbHkiOiJUZWtpbGUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSWzigJBIbyIsImZhbWlseSI6IktpbSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJKYWl5ZW9wIiwiZmFtaWx5IjoiTGVlIn19XSwiaWRzIjp7Imlzc24iOlsiMTIyNi0xMDI1IiwiMjAwNS05NjhYIl0sImlzYm4iOltdLCJkb2kiOiIxMC40NDkxL2Vlci4yMDE2LjAyOCIsIm1hZyI6IjI1MjM2Njk2NTciLCJvcGVuYWxleCI6IlcyNTIzNjY5NjU3In0sInRpdGxlIjoiRXh0ZW50IGFuZCBwZXJzaXN0ZW5jZSBvZiBkaXNzb2x2ZWQgb3h5Z2VuIGVuaGFuY2VtZW50IHVzaW5nIG5hbm9idWJibGVzIiwiYWJzdHJhY3QiOiJJbiB0aGlzIHN0dWR5LCBjaGFuZ2UgaW4gd2F0ZXItZGlzc29sdmVkIG94eWdlbiAoRE8pIHdhcyBhbmFseXplZCB1bmRlciB2YXJpb3VzIHN5bnRoZXRpYyB3YXRlciBxdWFsaXRpZXMgYW5kIG5hbm9idWJibGVzIChOQnMpIGFwcGxpY2F0aW9uIGNvbmRpdGlvbnMsIHN1Y2ggYXMgZ2FzIHR5cGUsIGluaXRpYWwgRE8gYXMgd2VsbCBhcyB3YXRlciBkaXNzb2x2ZWQsIHN1c3BlbmRlZCBhbmQgb3JnYW5pYyBtYXR0ZXJzIGNvbnRlbnRzLiBXaGVuIG94eWdlbiwgcmF0aGVyIHRoYW4gYWlyLCB3YXMgaW50cm9kdWNlZCBpbnRvIG5pdHJvZ2VuLWRlc29yYmVkIHVsdHJhLXB1cmUgd2F0ZXIsIHRoZSBzdGFnbmF0aW9uIHRpbWUgd2FzIHNpZ25pZmljYW50bHkgaW5jcmVhc2VkLiBJdCB0b29rIHRlbiBkYXlzIGZvciBETyBjb25jZW50cmF0aW9uIHRvIGRyb3AgYmFjayB0byBzYXR1cmF0aW9uLiBUaGUgaGlnaGVyIHRoZSBpbml0aWFsIERPIGNvbmNlbnRyYXRpb24sIHRoZSBsb25nZXIgcGFydGljbGVzIHdlcmUgb2JzZXJ2ZWQgYWJvdmUgc2F0dXJhdGlvbiBkdWUgdG8gcGFydGljbGUgc3RhYmlsaXR5IGltcHJvdmVtZW50LiBUaGUgb3h5Z2VuIG1hc3MgdHJhbnNmZXIgcmF0ZSBvZiAwLjA0ODIgbWcvTC9taW4gd2FzIGZvdW5kIHRvIHJlYWNoIGEgbWF4aW11bSBhdCBhbiBlbGVjdHJvbHl0aWMgY29uY2VudHJhdGlvbiBvZiAwLjc1IGcvTCwgYmV5b25kIHdoaWNoIHRoZSB0cmFuc2ZlciByYXRlIGRlY3JlYXNlZCBkdWUgdG8gYWRzb3JwdGlvbiBvZiBuZWdhdGl2ZSBpb25zIG9mIHRoZSBlbGVjdHJvbHl0ZSBhdCB0aGUgaW50ZXJmYWNlLiBIaWdoIGxldmVscyBvZiB0dXJiaWRpdHkgY2F1c2VkIGJ5IHN1c3BlbmRlZCBzb2xpZHMgaGF2ZSBiZWNvbWUgYSBiYXJyaWVyIHRvIGRpc3NvbHV0aW9uIG9mIE5CcyBveHlnZW4gaW50byB0aGUgd2F0ZXIgc29sdXRpb24sIGFuZCB0aHVzIGFmZmVjdGVkIHRoZSB0cmFuc2ZlciBwZXJmb3JtYW5jZS4gT24gdGhlIG90aGVyIGhhbmQsIGJ5IGFwcGx5aW5nIE5CcyBmb3IganVzdCBhbiBob3VyLCB1cCB0byA3LjIlIGRlZ3JhZGF0aW9uIG9mIGdsdWNvc2UgYXMgcmVwcmVzZW50YXRpdmUgb3JnYW5pYyBtYXR0ZXIgd2FzIGFjaGlldmVkLiBUaHVzLCBOQnMgdGVjaG5vbG9neSB3b3VsZCBtYWludGFpbiBhIGhpZ2ggRE8gZXh0ZW50IGZvciBhbiBleHRlbmRlZCBkdXJhdGlvbiwgYW5kIHRodXMgY2FuIGltcHJvdmUgd2F0ZXIgcXVhbGl0eSBwcm92aWRlZCB0aGF0IHdhdGVyIGNoZW1pc3RyeSBpcyBjbG9zZWx5IG1vbml0b3JlZCBkdXJpbmcgaXRzIGFwcGxpY2F0aW9uLiBLZXl3b3JkczogQXBwbGljYXRpb24gY29uZGl0aW9ucywgRGlzc29sdmVkIG94eWdlbiwgTmFub2J1YmJsZXMsIFdhdGVyIHF1YWxpdHkiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuNDQ5MS9lZXIuMjAxNi4wMjgiLCJyZXNvdXJjZVVybCI6Imh0dHA6Ly9lZWVyLm9yZy91cGxvYWQvZWVyLTIxLTQtNDI3LnBkZiIsImlzc3VlZERhdGUiOiIyMDE2LTA5LTEyIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiRW52aXJvbm1lbnRhbCBFbmdpbmVlcmluZyBSZXNlYXJjaCIsImlzc3VlIjo0LCJ2b2x1bWUiOjIxLCJpbXBhY3RGYWN0b3IiOiI0LjA1In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6NDI3LCJsYXN0UGFnZSI6NDM1fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IktvcmVhbiBTb2NpZXR5IG9mIEVudmlyb25tZW50YWwgRW5naW5lZXJpbmcifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjI0LCJjaXRlZEJ5Q291bnQiOjMzfX0sInF1b3RlIjoiVGhpcyB2ZXJpZmllcyB0aGUgcG9zc2liaWxpdHkgb2YgdGhlIGV4aXN0ZW5jZSAgb2YgTkJzLCBzaW5jZSBhdCBhIGhpZ2hlciBETyBjb25jZW50cmF0aW9uLCB0aGUgb3h5Z2VuIGRpZmZ1c2lvbiAgd291bGQgdGFrZSBtb3JlIHRpbWUuIFRoZSBoaWdoIGdhcyBjb25jZW50cmF0aW9uIGluIHRoZSBsaXF1aWQgIGV4cGxhaW5zIHRoZSBleHRlbnNpb24gb2YgTkIgc3RhYmlsaXR5LiAgVGhlIG1hZ25pdHVkZXMgb2YgemV0YSBwb3RlbnRpYWwgb2Ygb3h5Z2VuIG5hbm8tYnViYmxlZCB3YXRlciAgd2VyZSBwb3NzaWJseSBoaWdoZXIgdGhhbiB0aG9zZSBvZiBhaXIgbmFuby1idWJibGVkIHdhdGVyLiBBdCAgYSBoaWdoIG1hZ25pdHVkZSB6ZXRhIHBvdGVudGlhbCB2YWx1ZSwgdGhlIGVsZWN0cmljYWxseSBjaGFyZ2VkICBwYXJ0aWNsZXMgdGVuZCB0byByZXBlbCBlYWNoIG90aGVyLCBhdm9pZGluZyBhZ2dyZWdhdGlvbi4gRm9yIGJ1YmJsZSAgZGlzcGVyc2lvbiwgdGhlIGhpZ2ggemV0YSBwb3RlbnRpYWwgY291bGQgY3JlYXRlIHJlcHVsc2lvbiBmb3JjZXMgIHRoYXQgd291bGQgYXZvaWQgdGhlIGNvYWxlc2NlbmNlIG9mIGJ1YmJsZXMsIHRodXMgY29udHJpYnV0aW5nICB0byB0aGUgc3RhYmlsaXphdGlvbiBvZiB0aGUgYnViYmxlcy4gVGhlcmVmb3JlLCB0aGUgb3h5Z2VuIE5CcyAgY291bGQgYmUgc3RhYmlsaXplZCBpbiB3YXRlciBieSB0aGUgZWxlY3RyaWNhbGx5IGNoYXJnZWQgc3VyZmFjZXMsICB3aGlsZSB0aGUgbmFuby1idWJibGVkIGFpciB3b3VsZCBiZSBiZWxvdyB0aGUgbGltaXQgb2YgdGhlIHN0YWJpbGl6YS10aW9uIGJ5IHJlcHVsc2lvbiBmb3JjZXMuIEZ1cnRoZXJtb3JlLCB0aGUgYmlnIGRpZmZlcmVuY2UgYmV0d2VlbiB0aGUgcGVyaW9kcyBpbiB3aGljaCAgYnViYmxlcyBjb3VsZCBiZSBvYnNlcnZlZCBhcyBzdGFibGUgaW4gdGhlIG94eWdlbiBuYW5vLWJ1YmJsZWQgIHdhdGVyICgxMC0xNSBkKSBhbmQgaW4gdGhhdCBvZiBhaXIgbmFuby1idWJibGVkICgzLTUgZCkgaXMgbGlrZWx5IEZpZy4gNi4gR3JhcGhzIHNob3dpbmcgc3RhZ25hdGlvbiB0aW1lIGZvciBkaWZmZXJlbnQgaW5pdGlhbCBkaXNzb2x2ZWQgb3h5Z2VuIChETykuIHJlbGF0ZWQgdG8gdGhlIGxvd2VyIGRpc3NvbHV0aW9uIG9mIHRoZSBhaXIgaW4gd2F0ZXIgaW4gcmVsYXRpb24gdG8gdGhlIGRpc3NvbHV0aW9uIG9mIG94eWdlbi4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo2Ni4xNzg4MDI0OTAyMzQzOCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC43MTQ4ODk1MjYzNjcxOSwiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjY3LjU3Njc3NDU5NzE2Nzk3LCJsZWZ0Ijo3Ljg2NTU0NTI3MjgyNzE0OCwid2lkdGgiOjQwLjY4ODI3MDU2ODg0NzY1NiwiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjY4Ljk2NzIzMTc1MDQ4ODI4LCJsZWZ0Ijo3Ljg2NTU0NTI3MjgyNzE0OCwid2lkdGgiOjQwLjY1NzE4MDc4NjEzMjgxLCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6NzAuMzU3Njg4OTAzODA4NiwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjoyNS4zNDEyNTUxODc5ODgyOCwiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjcxLjc1NTY1MzM4MTM0NzY2LCJsZWZ0Ijo5LjU0OTY0MTYwOTE5MTg5NSwid2lkdGgiOjM5LjAyNDc4NDA4ODEzNDc2NiwiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjczLjE0NjEwMjkwNTI3MzQ0LCJsZWZ0Ijo3Ljg2NTU0NTI3MjgyNzE0OCwid2lkdGgiOjQwLjY1NDAyOTg0NjE5MTQwNiwiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjc0LjUzNjU2MDA1ODU5Mzc1LCJsZWZ0Ijo3Ljg2NTU0NTI3MjgyNzE0OCwid2lkdGgiOjQwLjY1MzQ2NTI3MDk5NjA5NCwiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjc1LjkzNDUzMjE2NTUyNzM0LCJsZWZ0Ijo3Ljg2NTU0NTI3MjgyNzE0OCwid2lkdGgiOjQwLjY3MzIzNjg0NjkyMzgzLCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6NzcuMzI0OTg5MzE4ODQ3NjYsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6NDAuNjQ5MzUzMDI3MzQzNzUsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3OC43MTU0NDY0NzIxNjc5NywibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NTQ2MjQ5Mzg5NjQ4NDQsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo4MC4xMTM0MTA5NDk3MDcwMywibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NTEwNTA1Njc2MjY5NSwiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjgxLjUwMzg2MDQ3MzYzMjgxLCJsZWZ0Ijo3Ljg2NTU0NTI3MjgyNzE0OCwid2lkdGgiOjQwLjY1NDM0MjY1MTM2NzE5LCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODIuODk0MzE3NjI2OTUzMTIsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6NDAuMzIxODIzMTIwMTE3MTksImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo4NC4yOTIyODk3MzM4ODY3MiwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjoxNS44NzAyOTU1MjQ1OTcxNjgsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo4NS42ODI3NDY4ODcyMDcwMywibGVmdCI6OS41NDk2NDE2MDkxOTE4OTUsIndpZHRoIjozOC45ODk3MDQxMzIwODAwOCwiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjg3LjA3MzIwNDA0MDUyNzM0LCJsZWZ0Ijo3Ljg2NTU0NTI3MjgyNzE0OCwid2lkdGgiOjQwLjcxODQ5NDQxNTI4MzIsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo4OC40NzExNjg1MTgwNjY0LCJsZWZ0Ijo3Ljg2NTU0NTI3MjgyNzE0OCwid2lkdGgiOjQwLjY4NzQxOTg5MTM1NzQyLCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6MzQuNTEzMjcxMzMxNzg3MTEsImxlZnQiOjUxLjkzMjc3NzQwNDc4NTE1Niwid2lkdGgiOjQwLjY0NzA1Mjc2NDg5MjU4LCJoZWlnaHQiOjIuNzE4NjY2NTUzNDk3MzE0NX19LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjM4LjcyNTE1ODY5MTQwNjI1LCJsZWZ0Ijo1MS45MzI3Nzc0MDQ3ODUxNTYsIndpZHRoIjo0MC42NDMxMzUwNzA4MDA3OCwiaGVpZ2h0IjoxLjI1MzI4ODUwNzQ2MTU0Nzl9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo0MC4xMTU2MTIwMzAwMjkzLCJsZWZ0Ijo1MS45MzI3Nzc0MDQ3ODUxNTYsIndpZHRoIjo0MC42NjQ1ODUxMTM1MjUzOSwiaGVpZ2h0IjoxLjI1MzI4ODUwNzQ2MTU0Nzl9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI0MzEifX1d.

Beyond direct oxygenation, nanobubbles improve feed conversion ratio and digestive efficiency through multifaceted biophysical effects. The collapse of nanobubbles generates localized shear forces and reactive oxygen species (ROS), which can enhance gut microbiota composition without compromising host health at sub-lethal doses. These ROS, including hydroxyl radicals (•OH) and superoxide (O₂•⁻), are produced from the extreme pressures during implosion, as described by the Young-Laplace equation (ΔP = 2σ/r), where smaller radii (r < 1 μm) amplify the stress (Hutagalung et al., 2023; Wu et al., 2021)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlN1dHJpc25vIFNhbG9tbyIsImZhbWlseSI6Ikh1dGFnYWx1bmcifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW5kZSBGdWRqYSIsImZhbWlseSI6IlJhZnJ5YW50byJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJXZWkiLCJmYW1pbHkiOiJTdW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTnVyb2NobWEiLCJmYW1pbHkiOiJKdWxpYXNpaCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTcmkiLCJmYW1pbHkiOiJBZGl0aWEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSml6aG91IiwiZmFtaWx5IjoiSmlhbmcifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQXJyYW1lbCIsImZhbWlseSI6IkFycmFtZWwifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSGVybWF3YW4gS3Jlc25vIiwiZmFtaWx5IjoiRGlwb2pvbm8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU3JpIEhhcmphdGkiLCJmYW1pbHkiOiJTdWhhcmRpIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik51cnVsIFRhdWZpcXUiLCJmYW1pbHkiOiJSb2NobWFuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkRlZGR5IiwiZmFtaWx5IjoiS3VybmlhZGkifX1dLCJpZHMiOnsiaXNzbiI6WyIyMjk2LTY2NVgiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzODkvZmVudnMuMjAyMy4xMTU0NzM5Iiwib3BlbmFsZXgiOiJXNDMyODExNDkyMyJ9LCJ0aXRsZSI6IkNvbWJpbmF0aW9uIG9mIG96b25lLWJhc2VkIGFkdmFuY2VkIG94aWRhdGlvbiBwcm9jZXNzIGFuZCBuYW5vYnViYmxlcyBnZW5lcmF0aW9uIHRvd2FyZCB0ZXh0aWxlIHdhc3Rld2F0ZXIgcmVjb3ZlcnkiLCJhYnN0cmFjdCI6IlRoZSBpbnRyaWNhdGUgbmF0dXJlIG9mIHZhcmlvdXMgdGV4dGlsZSBtYW51ZmFjdHVyaW5nIHByb2Nlc3NlcyBpbnRyb2R1Y2VzIGNvbG9yZWQgZHllcywgc3VyZmFjdGFudHMsIGFuZCB0b3hpYyBjaGVtaWNhbHMgdGhhdCBoYXZlIGJlZW4gaGFybWZ1bCB0byBlY29zeXN0ZW1zIGluIHJlY2VudCB5ZWFycy4gSGVyZSwgYSBjb21iaW5hdGlvbiBvem9uZS1iYXNlZCBhZHZhbmNlZCBveGlkYXRpb24gcHJvY2VzcyAoQU9QKSBpcyBjb3VwbGVkIHdpdGggYSBuYW5vYnViYmxlcyBnZW5lcmF0b3IgZm9yIHRoZSBnZW5lcmF0aW9uIG9mIG96b25lIG5hbm9idWJibGVzIChOQikgdXRpbGl6ZWQgdGhlIHNhbWUgdG8gdHJlYXQgdGhlIHByaW1hcnkgZWZmbHVlbnQgYWNxdWlyZWQgZnJvbSB0ZXh0aWxlIHdhc3Rld2F0ZXJzLiBIZXJlIHdlIGZpbmQgc2V2ZXJhbCBrZXkgcGFyYW1ldGVycyBzdWNoIGFzIGNoZW1pY2FsIG94eWdlbiBkZW1hbmQgYW1tb25pYSBjb250ZW50IChOSCAzICksIGFuZCB0b3RhbCBzdXNwZW5kZWQgc29saWRzIGluZGljYXRpbmcgYSBzdWJzdGFudGlhbCByZWNvdmVyeSBpbiB3aGljaCB0aGUgcmVzcGVjdGl2ZSBwZXJjZW50YWdlcyBvZiA4MS4xJSwgMzAuODElLCBhbmQgNDEuOTglLCB1cG9uIDMwMCBtaW4gcmVzaWRlbmNlIHRpbWUgYXJlIGFjaGlldmVkLiBPbiB0aGUgb3RoZXIgaGFuZCwgdGhlIHBIIGlzIHNoaWZ0ZWQgZnJvbSA3LjkzIHRvIDcuNDYsIGluZGljYXRpbmcgdGhlIGdlbmVyYXRpb24gb2YgaHlkcm9nZW4gcGVyb3hpZGUgKEggMiBPIDIgKSBkdWUgdG8gdGhlIHRlcm1pbmF0aW9uIHJlYWN0aW9uIGFuZCB0aGUgc2VsZi1yZWFjdGlvbiBvZiByZWFjdGl2ZSBveHlnZW4gc3BlY2llcyAoUk9TKS4gV2UgcHJvcG9zZSB0aGF0IHRoZSByZWFjdGl2ZSBveHlnZW4gc3BlY2llcyBjYW4gYmUgaWRlbnRpZmllZCBmcm9tIHRoZSBuZWdhdGl2ZSB6ZXRhIHBvdGVudGlhbCBtZWFzdXJlbWVudCAo4oiSMjIuNDMgwrEgMC4zNCBtVikgY29sbGVjdGVkIGluIHRoZSBmaW5hbCBzdGF0ZSBvZiB0cmVhdG1lbnQuIFRoZSBjb21iaW5lZCBtZXRob2QgaGFzIHN1Y2Nlc3NmdWxseSBnZW5lcmF0ZWQgb3pvbmUgbmFub2J1YmJsZXMgd2l0aCA5OS45NCUgb2Ygc2l6ZSBkaXN0cmlidXRlZCBpbiAyMTYuOSBubS4gVGhpcyBoaWdobGlnaHRzIHRoYXQgZW5oYW5jZW1lbnQgb2Ygb3pvbmXigJlzIHJlYWN0aXZpdHkgcGxheXMgYSBjcnVjaWFsIHJvbGUgaW4gaW1wcm92aW5nIHRoZSB3YXRlciBxdWFsaXR5IG9mIHRleHRpbGUgd2FzdGV3YXRlciB0b3dhcmRzIGJlaW5nIHRlY2hub2xvZ2ljYWxseSBlZmZpY2llbnQgdG8gZGF0ZS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMzM4OS9mZW52cy4yMDIzLjExNTQ3MzkiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vd3d3LmZyb250aWVyc2luLm9yZy9hcnRpY2xlcy8xMC4zMzg5L2ZlbnZzLjIwMjMuMTE1NDczOS9wZGYiLCJpc3N1ZWREYXRlIjoiMjAyMy0wMy0yMSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZyb250aWVycyBpbiBFbnZpcm9ubWVudGFsIFNjaWVuY2UiLCJ2b2x1bWUiOjExLCJpbXBhY3RGYWN0b3IiOiI1LjY0In0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkZyb250aWVycyBNZWRpYSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NjQsImNpdGVkQnlDb3VudCI6MTl9fSwicXVvdGUiOiJPdXIgZmluZGluZyByZWdhcmRpbmcgQ09EIHJlZHVjdGlvbiB0aGF0IG9wZXJhdGVkIGF0IG5ldXRyYWwgcEggaW4gYWdyZWVtZW50IHdpdGggdGhlIHByZXZpb3VzIHJlcG9ydHMgKEx1Y2FzIGV0IGFsLiwgMjAwOTsgVGFrYWhhc2hpIGV0IGFsLiwgMjAxMjsgWWFuZyBldCBhbC4sIDIwMTI7IFRlbWVzZ2VuIGV0IGFsLiwgMjAxNykuIFRoZSBzZWNvbmQgcmVjb3Zlcnkgc3RhZ2UgY29ycmVzcG9uZHMgdG8gdGhlIHV0aWxpemF0aW9uIG9mIG96b25lLWJhc2VkIEFPUCBhcyBpbmRpY2F0ZWQgYnkgdGhlIGJsdWUgYXJyb3cgaW4gRmlndXJlIDcuIEluIHRoaXMgc2VjdGlvbiwgdGhlIGdlbmVyYXRpb24gb2YgUk9TIG9yIG5lZ2F0aXZlbHktY2hhcmdlZCBOQiBkb21pbmFudGx5IGNvbnRyaWJ1dGVkIHRvIGRlZ3JhZGUgdGhlIG9yZ2FuaWMgY29udGFtaW5hbnRzLiBXZSByZXZpc2l0IHRoZSBjb252ZW50aW9uYWwgdGhlcm1vZHluYW1pY3Mgb2YgYnViYmxlIGNvbGxhcHNlIHBoZW5vbWVub24gdG8gZGVzY3JpYmUgdGhlIFJPUyBnZW5lcmF0aW9uIChHaGFkaW1raGFuaSBldCBhbC4sIDIwMTYpIGFuZCBvem9uZSBicmVha2Rvd24gKElraGxhcSBldCBhbC4sIDIwMTM7IE9oIGFuZCBOZ3V5ZW4sIDIwMjIpLiBJbiB0aGUgb3pvbmUgZGVjb21wb3NpdGlvbiwgd2UgYXR0cmlidXRlIHRoZSBST1MgaXMgYSBjaGFpbiByZWFjdGlvbiBjb21wcmlzZWQgb2YgaW5pdGlhdGlvbiwgcHJvcGFnYXRpb24sIGFuZCB0ZXJtaW5hdGlvbiAoS2h1bnRpYSBldCBhbC4sIDIwMTUpLiBGdXJ0aGVybW9yZSwgUk9TIHZpYSBjb2xsYXBzaW5nIGNhdml0eSBidWJibGVzIGNhbiBiZSBkZXNjcmliZWQgYmFzZWQgb24gdGhlIFlvdW5nLUxhcGxhY2UgZXF1YXRpb24gKEtpbXVyYSBldCBhbC4sIDIwMDQ7IFRha2FoYXNoaSBldCBhbC4sIDIwMDc7IEdoYWRpbWtoYW5pIGV0IGFsLiwgMjAxNikuIFAg77%2B9IFAxICsgNM%2BD77%2B9ZCAoMykgd2hlcmUgUCBkZW5vdGVzIGdhcyBwcmVzc3VyZSwgUGwgZGVub3RlcyBsaXF1aWQgcHJlc3N1cmUsIM%2BDIGRlbm90ZXMgbGlxdWlkIHN1cmZhY2UgdGVuc2lvbiwgYW5kIGQgZGVub3RlcyB0aGUgYnViYmxlIGRpYW1ldGVyLiBUaGUgaW50ZXJuYWwgcHJlc3N1cmUgb2YgYSBidWJibGUgaXMgc3Vic3RhbnRpYWxseSBkZXRlcm1pbmVkIGJ5IGl0cyBzaXplLCBhY2NvcmRpbmcgdG8gRXEuIDMuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NTkuMjc4ODkyNTE3MDg5ODQ0LCJsZWZ0Ijo4LjMzMzE3Mjc5ODE1NjczOCwid2lkdGgiOjQwLjQ3MTgzNjA5MDA4Nzg5LCJoZWlnaHQiOjEuMTc1NzgzNTE0OTc2NTAxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjYwLjY5OTczNzU0ODgyODEyNSwibGVmdCI6OC4zMzMxNzA4OTA4MDgxMDUsIndpZHRoIjo0MC40NzA5MzIwMDY4MzU5NCwiaGVpZ2h0IjoxLjE3NTc4MzUxNDk3NjUwMTV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo2Mi4xMjA1NzQ5NTExNzE4NzUsImxlZnQiOjguMzMzMTcwODkwODA4MTA1LCJ3aWR0aCI6NDAuNDcxMzgyMTQxMTEzMjgsImhlaWdodCI6MS4xNzU3ODM1MTQ5NzY1MDE1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NjMuNTM0NzE3NTU5ODE0NDUsImxlZnQiOjguMzMzMTcyNzk4MTU2NzM4LCJ3aWR0aCI6MTQuMTAyMTk2NjkzNDIwNDEsImhlaWdodCI6MS4xNzU3ODM1MTQ5NzY1MDE1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NjQuOTU1NTU4Nzc2ODU1NDcsImxlZnQiOjEwLjY3NTk4NDM4MjYyOTM5NSwid2lkdGgiOjM4LjEzMjk0MjE5OTcwNzAzLCJoZWlnaHQiOjEuMTc1NzgzNTE0OTc2NTAxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjY2LjM3NjQwMzgwODU5Mzc1LCJsZWZ0Ijo4LjMzMzE3NTY1OTE3OTY4OCwid2lkdGgiOjQwLjQ3MDkzMjAwNjgzNTk0LCJoZWlnaHQiOjEuMTc1NzgzNTE0OTc2NTAxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjY3Ljc5NzI0ODg0MDMzMjAzLCJsZWZ0Ijo4LjMzMzE3NzU2NjUyODMyLCJ3aWR0aCI6NC41NDQzMzI5ODExMDk2MTksImhlaWdodCI6MS4xNzU3ODM1MTQ5NzY1MDE1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NjcuNzk3MjQ4ODQwMzMyMDMsImxlZnQiOjE0LjE5MDIwMTc1OTMzODM3OSwid2lkdGgiOjEuODgxMjk3NzA3NTU3Njc4MiwiaGVpZ2h0IjoxLjE3NTc4MzUxNDk3NjUwMTV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo2Ny43OTcyNDg4NDAzMzIwMywibGVmdCI6MTcuMzcxMDg2MTIwNjA1NDcsIndpZHRoIjo2LjM3ODkzODE5ODA4OTYsImhlaWdodCI6MS4xNzU3ODM1MTQ5NzY1MDE1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NjcuNzk3MjQ4ODQwMzMyMDMsImxlZnQiOjI1LjA2NjAyNjY4NzYyMjA3LCJ3aWR0aCI6MS4xOTE0MzgzMTcyOTg4ODkyLCJoZWlnaHQiOjEuMTc1NzgzNTE0OTc2NTAxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjY3Ljc5NzI0ODg0MDMzMjAzLCJsZWZ0IjoyNy41NjEyNzE2Njc0ODA0Nywid2lkdGgiOjIuNzQyODY4MTg1MDQzMzM1LCJoZWlnaHQiOjEuMTc1NzgzNTE0OTc2NTAxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjY3Ljc5NzI0ODg0MDMzMjAzLCJsZWZ0IjozMS42MDg4NDY2NjQ0Mjg3MSwid2lkdGgiOjEuMzAxMzk0MjI0MTY2ODcwMSwiaGVpZ2h0IjoxLjE3NTc4MzUxNDk3NjUwMTV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo2Ny43OTcyNDg4NDAzMzIwMywibGVmdCI6MzQuMjE4NDE0MzA2NjQwNjI1LCJ3aWR0aCI6MTEuMjg0NzcxOTE5MjUwNDg4LCJoZWlnaHQiOjEuMTc1NzgzNTE0OTc2NTAxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjY3Ljc5NzI0ODg0MDMzMjAzLCJsZWZ0Ijo0Ni44MTc5ODU1MzQ2Njc5Nywid2lkdGgiOjEuOTkxMjUzNDk1MjE2MzY5NiwiaGVpZ2h0IjoxLjE3NTc4MzUxNDk3NjUwMTV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo2OS4yMTgwOTM4NzIwNzAzMSwibGVmdCI6OC4zMzMxNzc1NjY1MjgzMiwid2lkdGgiOjQwLjQ3Mzc5MzAyOTc4NTE1NiwiaGVpZ2h0IjoxLjE3NTc4MzUxNDk3NjUwMTV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo3MC42Mzg5Mzg5MDM4MDg2LCJsZWZ0Ijo4LjMzMzE3NzU2NjUyODMyLCJ3aWR0aCI6NDAuNDcxOTg4Njc3OTc4NTE2LCJoZWlnaHQiOjEuMTc1NzgzNTE0OTc2NTAxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjcyLjA1OTc3NjMwNjE1MjM0LCJsZWZ0Ijo4LjMzMzE3NzU2NjUyODMyLCJ3aWR0aCI6NDAuNDY2NDExNTkwNTc2MTcsImhlaWdodCI6MS4xNzU3ODM1MTQ5NzY1MDE1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NzMuNDgwNjIxMzM3ODkwNjIsImxlZnQiOjguMzMzMTc3NTY2NTI4MzIsIndpZHRoIjo0MC40NzU1OTczODE1OTE4LCJoZWlnaHQiOjEuMTc1NzgzNTE0OTc2NTAxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjc0Ljg5NDc1MjUwMjQ0MTQsImxlZnQiOjguMzMzMTc3NTY2NTI4MzIsIndpZHRoIjo0MC40NzIyOTM4NTM3NTk3NjYsImhlaWdodCI6MS4xNzU3ODM1MTQ5NzY1MDE1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NzYuMzE1NTk3NTM0MTc5NjksImxlZnQiOjguMzMzMTc4NTIwMjAyNjM3LCJ3aWR0aCI6MC42NTgyMjgwOTkzNDYxNjA5LCJoZWlnaHQiOjEuMTc1NzgzNTE0OTc2NTAxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjc2LjMxNTU5NzUzNDE3OTY5LCJsZWZ0IjoxMC4xOTk4NjUzNDExODY1MjMsIndpZHRoIjoxNi44NTUxNTU5NDQ4MjQyMiwiaGVpZ2h0IjoxLjE3NTc4MzUxNDk3NjUwMTV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo3Ni4zMTU1OTc1MzQxNzk2OSwibGVmdCI6MjguMjY2MDM4ODk0NjUzMzIsIndpZHRoIjoxLjE5MTQzODMxNzI5ODg4OTIsImhlaWdodCI6MS4xNzU3ODM1MTQ5NzY1MDE1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NzYuMzE1NTk3NTM0MTc5NjksImxlZnQiOjMwLjY2NTkzNzQyMzcwNjA1NSwid2lkdGgiOjUuODQ1NzI3OTIwNTMyMjI3LCJoZWlnaHQiOjEuMTc1NzgzNTE0OTc2NTAxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjc2LjMxNTU5NzUzNDE3OTY5LCJsZWZ0IjozNy43MjI4MzkzNTU0Njg3NSwid2lkdGgiOjcuNjE3MDY5NzIxMjIxOTI0LCJoZWlnaHQiOjEuMTc1NzgzNTE0OTc2NTAxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjc2LjMxNTU5NzUzNDE3OTY5LCJsZWZ0Ijo0Ni41NTEwNzg3OTYzODY3Miwid2lkdGgiOjIuMjU2MzUxMjMyNTI4Njg2NSwiaGVpZ2h0IjoxLjE3NTc4MzUxNDk3NjUwMTV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo3Ny43MzY0NTAxOTUzMTI1LCJsZWZ0Ijo4LjMzMzE3ODUyMDIwMjYzNywid2lkdGgiOjIxLjA4MzAyNjg4NTk4NjMyOCwiaGVpZ2h0IjoxLjE3NTc4MzUxNDk3NjUwMTV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo3OS4xNTcyOTUyMjcwNTA3OCwibGVmdCI6MTAuNjc1OTg3MjQzNjUyMzQ0LCJ3aWR0aCI6MzguMTI5OTIwOTU5NDcyNjU2LCJoZWlnaHQiOjEuMTc1NzgzNTE0OTc2NTAxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjgwLjU3ODE0MDI1ODc4OTA2LCJsZWZ0Ijo4LjMzMzAyNzgzOTY2MDY0NSwid2lkdGgiOjQwLjQ3Nzg1NTY4MjM3MzA1LCJoZWlnaHQiOjEuMTc1NzgzNTE0OTc2NTAxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjgxLjk5ODk3NzY2MTEzMjgxLCJsZWZ0Ijo4LjMzMzAyNjg4NTk4NjMyOCwid2lkdGgiOjM4LjE0NzU0NDg2MDgzOTg0NCwiaGVpZ2h0IjoxLjE3NTc4MzUxNDk3NjUwMTV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo4My4xMDEzNDEyNDc1NTg2LCJsZWZ0IjoyNC4yMDkyNzA0NzcyOTQ5MjIsIndpZHRoIjoyNC42MDI1OTI0NjgyNjE3MiwiaGVpZ2h0Ijo1LjM0NjQwNjQ1OTgwODM1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6ODYuMTgxMzgxMjI1NTg1OTQsImxlZnQiOjguMzMzMjU0ODE0MTQ3OTUsIndpZHRoIjo0MC40NzQ0MTQ4MjU0Mzk0NSwiaGVpZ2h0IjoxLjM1NzYwNzk2MDcwMDk4ODh9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo4Ny42MDIyMjYyNTczMjQyMiwibGVmdCI6OC4zMzMyNzc3MDIzMzE1NDMsIndpZHRoIjo0MC40NzQ1NDQ1MjUxNDY0ODQsImhlaWdodCI6MS4xNzU3ODM1MTQ5NzY1MDE1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6ODkuMDIzMDc4OTE4NDU3MDMsImxlZnQiOjguMzMzNDMwMjkwMjIyMTY4LCJ3aWR0aCI6NDAuNDcyNzQwMTczMzM5ODQ0LCJoZWlnaHQiOjEuMTc1NzgzNTE0OTc2NTAxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjkwLjQ0MzkyMzk1MDE5NTMxLCJsZWZ0Ijo4LjMzMzQzMDI5MDIyMjE2OCwid2lkdGgiOjQwLjQ3NjgwNjY0MDYyNSwiaGVpZ2h0IjoxLjE3NTc4MzUxNDk3NjUwMTV9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI2In19LHsib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjMwLCJzdWJ0eXBlIjp7fSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSmlhamlhIiwiZmFtaWx5IjoiV3UifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiS2VqaWEiLCJmYW1pbHkiOiJaaGFuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJDaGVuZyIsImZhbWlseSI6IkNlbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJYaWFvZ2FuZyIsImZhbWlseSI6Ild1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlJ1eWluIiwiZmFtaWx5IjoiTWFvIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Illpbmd5aW5nIiwiZmFtaWx5IjoiWmhlbmcifX1dLCJpZHMiOnsiaXNzbiI6WyIyMTkxLTA4NTUiXSwiaXNibiI6W10sImRvaSI6IjEwLjExODYvczEzNTY4LTAyMS0wMTI1NC0wIiwibWFnIjoiMzE3NTY4MzU2MSIsInBtaWQiOiIzNDE4NDEzNyIsInBtY2lkIjoiODIzOTEwOSIsIm9wZW5hbGV4IjoiVzMxNzU2ODM1NjEifSwidGl0bGUiOiJSb2xlIG9mIGJ1bGsgbmFub2J1YmJsZXMgaW4gcmVtb3Zpbmcgb3JnYW5pYyBwb2xsdXRhbnRzIGluIHdhc3Rld2F0ZXIgdHJlYXRtZW50IiwiYWJzdHJhY3QiOiJBYnN0cmFjdCBUaGUgb2NjdXJyZW5jZSBvZiBhIHZhcmlldHkgb2Ygb3JnYW5pYyBwb2xsdXRhbnRzIGhhcyBjb21wbGljYXRlZCB3YXN0ZXdhdGVyIHRyZWF0bWVudDsgdGh1cywgdGhlIHNlYXJjaCBmb3Igc3VzdGFpbmFibGUgYW5kIGVmZmVjdGl2ZSB0cmVhdG1lbnQgdGVjaG5vbG9neSBoYXMgZHJhd24gc2lnbmlmaWNhbnQgYXR0ZW50aW9uLiBJbiByZWNlbnQgeWVhcnMsIGJ1bGsgbmFub2J1YmJsZXMsIHdoaWNoIGhhdmUgZXh0cmFvcmRpbmFyeSBwcm9wZXJ0aWVzIGRpZmZlcmluZyBmcm9tIHRob3NlIG9mIG1pY3JvYnViYmxlcywgaW5jbHVkaW5nIGhpZ2ggc3RhYmlsaXR5IGFuZCBsb25nIHJlc2lkZW5jZSB0aW1lcyBpbiB3YXRlciwgbGFyZ2Ugc3BlY2lmaWMgc3VyZmFjZSBhcmVhcywgaGlnaCBnYXMgdHJhbnNmZXIgZWZmaWNpZW5jeSBhbmQgaW50ZXJmYWNlIHBvdGVudGlhbCwgYW5kIHRoZSBjYXBhYmlsaXR5IHRvIGdlbmVyYXRlIGZyZWUgcmFkaWNhbHMsIGhhdmUgc2hvd24gYXR0cmFjdGl2ZSB0ZWNobm9sb2dpY2FsIGFkdmFudGFnZXMgYW5kIHByb21pc2luZyBhcHBsaWNhdGlvbiBwcm9zcGVjdHMgZm9yIHdhc3Rld2F0ZXIgdHJlYXRtZW50LiBJbiB0aGlzIHJldmlldywgdGhlIGJhc2ljIGNoYXJhY3RlcmlzdGljcyBvZiBidWxrIG5hbm9idWJibGVzIGFyZSBzdW1tYXJpemVkIGluIGRldGFpbCwgYW5kIHJlY2VudCBmaW5kaW5ncyByZWxhdGVkIHRvIHRoZWlyIGltcGxlbWVudGF0aW9uIHBhdGh3YXlzIGFuZCBtZWNoYW5pc21zIGluIG9yZ2FuaWMgd2FzdGV3YXRlciB0cmVhdG1lbnQgYXJlIHN5c3RlbWF0aWNhbGx5IGRpc2N1c3NlZCwgd2hpY2ggaW5jbHVkZXMgaW1wcm92aW5nIHRoZSBhaXIgZmxvdGF0aW9uIHByb2Nlc3MsIGluY3JlYXNpbmcgd2F0ZXIgYWVyYXRpb24gdG8gcHJvbW90ZSBhZXJvYmljIGJpb2xvZ2ljYWwgdGVjaG5vbG9naWVzIGluY2x1ZGluZyBiaW9sb2dpY2FsIGFjdGl2YXRlZCBjYXJib24sIGFjdGl2YXRlZCBzbHVkZ2UsIGFuZCBtZW1icmFuZSBiaW9yZWFjdG9ycywgYW5kIGdlbmVyYXRpbmcgYWN0aXZlIGZyZWUgcmFkaWNhbHMgdGhhdCBveGlkaXNlIG9yZ2FuaWMgY29tcG91bmRzLiBGaW5hbGx5LCB0aGUgY3VycmVudCB0ZWNobm9sb2dpY2FsIGRpZmZpY3VsdGllcyBvZiBidWxrIG5hbm9idWJibGVzIGFyZSBhbmFseXNlZCwgYW5kIGZ1dHVyZSBmb2N1cyBhcmVhcyBmb3IgcmVzZWFyY2ggb24gYnVsayBuYW5vYnViYmxlIHRlY2hub2xvZ3kgYXJlIGFsc28gcHJvcG9zZWQuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjExODYvczEzNTY4LTAyMS0wMTI1NC0wIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2FtYi1leHByZXNzLnNwcmluZ2Vyb3Blbi5jb20vY291bnRlci9wZGYvMTAuMTE4Ni9zMTM1NjgtMDIxLTAxMjU0LTAiLCJpc3N1ZWREYXRlIjoiMjAyMS0wNi0yOCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkFNQiBFeHByZXNzIiwiaXNzdWUiOjEsInZvbHVtZSI6MTEsImltcGFjdEZhY3RvciI6IjQuMjYifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiU3ByaW5nZXIgTmF0dXJlIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo3NiwiY2l0ZWRCeUNvdW50Ijo3NX19LCJxdW90ZSI6IkR1cmluZyB0aGUgc2hyaW5rYWdlIG9mIG5hbm9idWJibGVzLCB0aGUgZWxlYy10cmljIGNoYXJnZSBkZW5zaXR5IGFjY3VtdWxhdGVkIGluIHRoZSBlbGVjdHJpYyBkb3VibGUgIGxheWVyIGluY3JlYXNlcyByYXBpZGx5LiBCZWNhdXNlIG9mIHRoZSBkcmFzdGljIGNoYW5nZSBpbiAgdGhlIGdhc%2BKAk2xpcXVpZCBpbnRlcmZhY2UgYXQgdGhlIG1vbWVudCBvZiBidWJibGUgY29sLWxhcHNlLCB0aGUgaGlnaCBjb25jZW50cmF0aW9uIG9mIGlvbnMgYWdncmVnYXRlZCBvbiB0aGUgIGludGVyZmFjZSBpbnN0YW50bHkgcmVsZWFzZXMgdGhlIGFjY3VtdWxhdGVkIGNoZW1pLWNhbCBlbmVyZ3kuIE1lYW53aGlsZSB0aGUgZW50cmFwcGVkIHdhdGVyIG1vbGVjdWxlcyAgYXJlIHN1YmplY3RlZCB0byBleHRyZW1lbHkgaGlnaCB0ZW1wZXJhdHVyZXMgYW5kIHByZXMtc3VyZXMgd2hlbiB0aGV5IGFyZSBmb3JjZWZ1bGx5IGNvbXByZXNzZWQgYnkgZHluYW1pYyAgc3RpbXVsaSAoS3Jpc2huYW4gZXQgYWwuIDIwMDYpLCB3aGljaCByZXN1bHRzIGluIGNsZWF2YWdlICB0aGF0IHByb2R1Y2VzIHZhcmlvdXMgcmVhY3RpdmUgb3h5Z2VuIHNwZWNpZXMgKFJPU3MpLCAgaW5jbHVkaW5nIGh5ZHJveHlsIHJhZGljYWxzICjCt09IKSwgc3VwZXJveGlkZSBhbmlvbiAgcmFkaWNhbHMgKE8ywrfiiJIpLCBhbmQgc2luZ2xldCBveHlnZW4gKDFPMikgKEF0a2luc29uIGV0IGFsLiAgMjAxOTsgTGl1IGV0IGFsLiAyMDE1OyBMeXUgZXQgYWwuIDIwMTkpLiBTZXZlcmFsIHN0dWRpZXMgaGF2ZSAgYXBwbGllZCBlbGVjdHJvbiBzcGluIHJlc29uYW5jZSBvciB0aGUgYWN0aW9uIG9mIGZyZWUgcmFkaS1jYWwgaW5oaWJpdG9ycyB0byB2ZXJpZnkgdGhlaXIgZXhpc3RlbmNlLiBNaWNoYWlsaWRpIGV0IGFsLiAgKDIwMjApIHVzZWQgZWxlY3Ryb24gcGFyYW1hZ25ldGljIHJlc29uYW5jZSBiYXNlZCBvbiAgNSw1LWRpbWV0aHlsLTEtcHlycm9saW5lIE4tb3hpZGUgKERNUE8pIHNwaW4gdHJhcHBpbmcgIHRvIHF1YW50aXRhdGl2ZWx5IGFuYWx5c2UgZnJlZSByYWRpY2FscyBpbiBhaXItbmFub2J1YmJsZSAgYW5kIG94eWdlbi1uYW5vYnViYmxlIHN5c3RlbXMuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjozLCJiYm94Ijp7InRvcCI6MzUuNTQyMTY3NjYzNTc0MjIsImxlZnQiOjkuNTIzODAwODQ5OTE0NTUsIndpZHRoIjozOS4yODU3MDkzODExMDM1MTYsImhlaWdodCI6MS44Mzk0NzU4NzAxMzI0NDYzfX0seyJwYWdlTnVtYmVyIjozLCJiYm94Ijp7InRvcCI6MzcuMDYxMzA5ODE0NDUzMTI1LCJsZWZ0Ijo5LjUyMzgwMDg0OTkxNDU1LCJ3aWR0aCI6MzkuNjcyOTE2NDEyMzUzNTE2LCJoZWlnaHQiOjEuODM3NjU0NTkwNjA2Njg5NX19LHsicGFnZU51bWJlciI6MywiYmJveCI6eyJ0b3AiOjM4LjU3ODAyNTgxNzg3MTA5NCwibGVmdCI6OS41MjM4MDA4NDk5MTQ1NSwid2lkdGgiOjM5LjY3MzkxMjA0ODMzOTg0NCwiaGVpZ2h0IjoxLjgzNzY1NDU5MDYwNjY4OTV9fSx7InBhZ2VOdW1iZXIiOjMsImJib3giOnsidG9wIjo0MC4wOTQ3NDk0NTA2ODM1OTQsImxlZnQiOjkuNTIzODAwODQ5OTE0NTUsIndpZHRoIjozOS4yODU3MDkzODExMDM1MTYsImhlaWdodCI6MS44Mzg4NjYyMzM4MjU2ODM2fX0seyJwYWdlTnVtYmVyIjozLCJiYm94Ijp7InRvcCI6NDEuNjEzMjg1MDY0Njk3MjY2LCJsZWZ0Ijo5LjUyMzgwMDg0OTkxNDU1LCJ3aWR0aCI6MzkuNjczNDE2MTM3Njk1MzEsImhlaWdodCI6MS44Mzc2NTQ1OTA2MDY2ODk1fX0seyJwYWdlTnVtYmVyIjozLCJiYm94Ijp7InRvcCI6NDMuMTMwMDAxMDY4MTE1MjM0LCJsZWZ0Ijo5LjUyMzgwMDg0OTkxNDU1LCJ3aWR0aCI6MzkuMjg1NzA5MzgxMTAzNTE2LCJoZWlnaHQiOjEuODM4MjYwNTMxNDI1NDc2fX0seyJwYWdlTnVtYmVyIjozLCJiYm94Ijp7InRvcCI6NDQuNjQ3OTMzOTU5OTYwOTQsImxlZnQiOjkuNTIzODAwODQ5OTE0NTUsIndpZHRoIjozOS42NzM1Njg3MjU1ODU5NCwiaGVpZ2h0IjoxLjgzNzY1NDU5MDYwNjY4OTV9fSx7InBhZ2VOdW1iZXIiOjMsImJib3giOnsidG9wIjo0Ni4xNjQ2NDk5NjMzNzg5MDYsImxlZnQiOjkuNTIzODAwODQ5OTE0NTUsIndpZHRoIjozOS4yODU3MDkzODExMDM1MTYsImhlaWdodCI6MS44MzgyNjA1MzE0MjU0NzZ9fSx7InBhZ2VOdW1iZXIiOjMsImJib3giOnsidG9wIjo0Ny42ODI1NzkwNDA1MjczNDQsImxlZnQiOjkuNTIzODAwODQ5OTE0NTUsIndpZHRoIjozOS42NzcyMDQxMzIwODAwOCwiaGVpZ2h0IjoxLjgzNzY1NDU5MDYwNjY4OTV9fSx7InBhZ2VOdW1iZXIiOjMsImJib3giOnsidG9wIjo0OS4xOTkyOTg4NTg2NDI1OCwibGVmdCI6OS41MjM4MDA4NDk5MTQ1NSwid2lkdGgiOjM5LjY2OTQ2NDExMTMyODEyNSwiaGVpZ2h0IjoxLjgzNzY1NDU5MDYwNjY4OTV9fSx7InBhZ2VOdW1iZXIiOjMsImJib3giOnsidG9wIjo1MC43MTYwMTEwNDczNjMyOCwibGVmdCI6OS41MjM4MDA4NDk5MTQ1NSwid2lkdGgiOjM5LjY3NTU1NjE4Mjg2MTMzLCJoZWlnaHQiOjEuODM3NjU0NTkwNjA2Njg5NX19LHsicGFnZU51bWJlciI6MywiYmJveCI6eyJ0b3AiOjUyLjIzMjczMDg2NTQ3ODUxNiwibGVmdCI6OS41MjM4MDA4NDk5MTQ1NSwid2lkdGgiOjM5LjY3MzA4MDQ0NDMzNTk0LCJoZWlnaHQiOjIuMzU5MzM4OTk4Nzk0NTU1N319LHsicGFnZU51bWJlciI6MywiYmJveCI6eyJ0b3AiOjUzLjYwOTM3ODgxNDY5NzI2NiwibGVmdCI6OS41MjM4MDA4NDk5MTQ1NSwid2lkdGgiOjM5LjY3Mjg4MjA4MDA3ODEyNSwiaGVpZ2h0IjoyLjUwMTgzODkyMjUwMDYxMDR9fSx7InBhZ2VOdW1iZXIiOjMsImJib3giOnsidG9wIjo1NS4yNjg1OTI4MzQ0NzI2NTYsImxlZnQiOjkuNTI0NTkwNDkyMjQ4NTM1LCJ3aWR0aCI6MzkuNjY5NDY0MTExMzI4MTI1LCJoZWlnaHQiOjEuODM3NjU0NTkwNjA2Njg5NX19LHsicGFnZU51bWJlciI6MywiYmJveCI6eyJ0b3AiOjU2Ljc4NTMwODgzNzg5MDYyNSwibGVmdCI6OS41MjQ1OTA0OTIyNDg1MzUsIndpZHRoIjozOS4yODQ5MTk3Mzg3Njk1MywiaGVpZ2h0IjoxLjgzODg2NjIzMzgyNTY4MzZ9fSx7InBhZ2VOdW1iZXIiOjMsImJib3giOnsidG9wIjo1OC4zMDM4NDQ0NTE5MDQzLCJsZWZ0Ijo5LjUyMzgwMDg0OTkxNDU1LCJ3aWR0aCI6MzkuNjcwOTU5NDcyNjU2MjUsImhlaWdodCI6MS44Mzc2NTQ1OTA2MDY2ODk1fX0seyJwYWdlTnVtYmVyIjozLCJiYm94Ijp7InRvcCI6NTkuODIwNTY0MjcwMDE5NTMsImxlZnQiOjkuNTIzODAwODQ5OTE0NTUsIndpZHRoIjozOS42NzUzOTIxNTA4Nzg5MDYsImhlaWdodCI6MS44Mzc2NTQ1OTA2MDY2ODk1fX0seyJwYWdlTnVtYmVyIjozLCJiYm94Ijp7InRvcCI6NjEuMzM3MjgwMjczNDM3NSwibGVmdCI6OS41MjM4MDA4NDk5MTQ1NSwid2lkdGgiOjM5LjY3MDQ1MjExNzkxOTkyLCJoZWlnaHQiOjEuODM3NjU0NTkwNjA2Njg5NX19LHsicGFnZU51bWJlciI6MywiYmJveCI6eyJ0b3AiOjYyLjg1NDAwMzkwNjI1LCJsZWZ0Ijo5LjUyMzgwMDg0OTkxNDU1LCJ3aWR0aCI6MzkuNjcwNDM2ODU5MTMwODYsImhlaWdodCI6MS44Mzc2NTQ1OTA2MDY2ODk1fX0seyJwYWdlTnVtYmVyIjozLCJiYm94Ijp7InRvcCI6NjQuMzcwNzEyMjgwMjczNDQsImxlZnQiOjkuNTIzODAwODQ5OTE0NTUsIndpZHRoIjozOS42NzA2MDg1MjA1MDc4MSwiaGVpZ2h0IjoxLjgzNzY1NDU5MDYwNjY4OTV9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI0In19XQ%3D%3D. In P. vannamei cultures, nanobubble integration reduced FCR by up to 20%, correlating with elevated activities of digestive enzymes like amylase and protease, fostering faster biomass accumulation (Susanti et al., 2021)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkxpbHkiLCJmYW1pbHkiOiJTdXNhbnRpIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlN1eXVkIFdhcm5vIiwiZmFtaWx5IjoiVXRvbW8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTm92ZXJpdGEgRGlhbiIsImZhbWlseSI6IlRha2FyaW5hIn19XSwiaWRzIjp7Imlzc24iOlsiMjExNy00NDU4IiwiMjI3My0xNzA5Il0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDUxL2Jpb2NvbmYvMjAyMTMzMDUwMDUiLCJtYWciOiIzMTkzNTYzNzAzIiwib3BlbmFsZXgiOiJXMzE5MzU2MzcwMyJ9LCJ0aXRsZSI6IlN1c3RhaW5hYmlsaXR5IGFuZCBmZWFzaWJpbGl0eSBhc3Nlc3NtZW50cyBvZiBuYW5vYnViYmxlIGFlcmF0aW9uIHRlY2hub2xvZ3kgaW4gZWNvbm9taWMtc29jaW8gZW52aXJvbm1lbnQgb2YgUGVuYWV1cyB2YW5uYW1laSBzaHJpbXAgZmFybWluZyIsImFic3RyYWN0IjoiUGVuYWV1cyB2YW5uYW1laSBzaHJpbXAgZmFybWluZyBpbiBJbmRvbmVzaWEgaXMgZmFjZWQgd2l0aCBzZXZlcmFsIGNoYWxsZW5nZXMgaW4gdGhlIGVudmlyb25tZW50YWwsIGVjb25vbWljLCBhbmQgc29jaWFsIGFzcGVjdHMuIFRoZXJlZm9yZSwgdGhpcyBzdHVkeSBhaW1zIHRvIGFzc2VzcyB0aGUgYmVuZWZpdHMgb2Ygbm92ZWwgbmFub2J1YmJsZSBhZXJhdGlvbiBzeXN0ZW1zIGZvciB0aGUgZWNvc3lzdGVtLCBidXNpbmVzc2VzLCBhbmQgY29tbXVuaXRpZXMuIFRoaXMgaXMgYW4gZXhwZXJpbWVudGFsIHN0dWR5IGNvbmR1Y3RlZCBieSBjb21wYXJpbmcgUC4gdmFubmFtZWkgcG9zdCBsYXJ2YSAxMCByZWFyZWQgYXQgYSBkZW5zaXR5IG9mIDIwMDAgaW5kcy4vTCBpbiBhIHBvbmQgdHJlYXRlZCB3aXRoIG5hbm9idWJibGUgYW5kIHdpdGhvdXQgdHJlYXRtZW50cyAoY29udHJvbCkuIEZ1cnRoZXJtb3JlLCB0aGUgTmV0IFByZXNlbnQgVmFsdWUgKE5QVikgYW5kIHBheWJhY2sgcGVyaW9kIHdlcmUgY2FsY3VsYXRlZCB0byBhc3Nlc3MgdGhlIGVjb25vbWljIGZlYXNpYmlsaXR5IG9mIG5hbm9idWJibGUsIHdoaWxlIGNvbW11bml0eSBpbnRlcnZpZXdzIHdlcmUgdXNlZCBmb3IgdGhlIHNvY2lhbCBzZWN0b3JzLiBCYXNlZCBvbiB0aGUgcmVzdWx0cywgZW52aXJvbm1lbnRhbCB3YXN0ZXMgb2Ygc2hyaW1wIGZhcm1pbmcgaW4gZm9ybSBvZiB0b3RhbCBhbW1vbmlhIG5pdHJvZ2VuIChUQU4pIHdlcmUgcmVkdWNlZCBieSA5JSBmcm9tIDIuNTggbWcvbCAoOTUlQ0k6IDAsOTEsIDQsMjUpIGluIGNvbnRyb2wgY29tcGFyZWQgdG8gMi4zNSBtZy9sICg5NSVDSTogMCw4NiwgMyw4NCkgaW4gdHJlYXRtZW50LiBGdXJ0aGVybW9yZSwgdGhlIHJldmVudWVzIGZyb20gcG9zdCBsYXJ2YSBzYWxlcyBmb3IgZml2ZSB5ZWFycyBhbmQgbmFub2J1YmJsZSBpbnZlc3RtZW50IGNvc3RzIHNob3dlZCB0aGF0IHRoZSBlc3RpbWF0ZWQgTlBWIHdhcyBJRFIgNjQsODI0LDM3NCB3aXRoIGEgcGF5YmFjayBwZXJpb2Qgb2YgMS43IHllYXJzLiBUaGUgaW50ZXJ2aWV3cyBvbiB0cmFkaXRpb25hbCBzaHJpbXAgZmFybWluZyBzaG93ZWQgdGhhdCA2MS4xJSB0byA3Mi4yJSBvZiBjb21tdW5pdHkgbWVtYmVycyBhZ3JlZWQgb24gdGhlIHVzZSBvZiBuYW5vYnViYmxlIGFlcmF0aW9uIHRvIHN1cHBvcnQgbGl2ZWxpaG9vZHMuIFRoZXJlZm9yZSwgdGhlIHVzZSBvZiBuYW5vYnViYmxlcyBpcyBmZWFzaWJsZSB0byBzdXBwb3J0IHN1c3RhaW5hYmxlIFAuIHZhbm5hbWVpIGZhcm1pbmcuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjEwNTEvYmlvY29uZi8yMDIxMzMwNTAwNSIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cuYmlvLWNvbmZlcmVuY2VzLm9yZy9hcnRpY2xlcy9iaW9jb25mL3BkZi8yMDIxLzA1L2Jpb2NvbmZfaWNhdmVzczIwMjFfMDUwMDUucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjEtMDEtMDEiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA5IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJCSU8gV2ViIG9mIENvbmZlcmVuY2VzIiwidm9sdW1lIjozMywiaW1wYWN0RmFjdG9yIjoiMC42MiJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjUwMDUsImxhc3RQYWdlIjo1MDA1fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkVEUCBTY2llbmNlcyJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MTEsImNpdGVkQnlDb3VudCI6NH19LCJxdW90ZSI6IkFic3RyYWN0LiBQZW5hZXVzIHZhbm5hbWVpIHNocmltcCBmYXJtaW5nIGluIEluZG9uZXNpYSBpcyBmYWNlZCB3aXRoIHNldmVyYWwgY2hhbGxlbmdlcyBpbiB0aGUgZW52aXJvbm1lbnRhbCwgZWNvbm9taWMsIGFuZCBzb2NpYWwgYXNwZWN0cy4gIFRoZXJlZm9yZSwgdGhpcyBzdHVkeSBhaW1zIHRvIGFzc2VzcyB0aGUgYmVuZWZpdHMgb2Ygbm92ZWwgbmFub2J1YmJsZSAgYWVyYXRpb24gc3lzdGVtcyBmb3IgdGhlIGVjb3N5c3RlbSwgYnVzaW5lc3NlcywgYW5kIGNvbW11bml0aWVzLiBUaGlzIGlzIGFuICBleHBlcmltZW50YWwgc3R1ZHkgY29uZHVjdGVkIGJ5IGNvbXBhcmluZyBQLiB2YW5uYW1laSBwb3N0IGxhcnZhIDEwICByZWFyZWQgYXQgYSBkZW5zaXR5IG9mIDIwMDAgaW5kcy4vTCBpbiBhIHBvbmQgdHJlYXRlZCB3aXRoIG5hbm9idWJibGUgYW5kICB3aXRob3V0IHRyZWF0bWVudHMgKGNvbnRyb2wpLiBGdXJ0aGVybW9yZSwgdGhlIE5ldCBQcmVzZW50IFZhbHVlIChOUFYpIGFuZCAgcGF5YmFjayBwZXJpb2Qgd2VyZSBjYWxjdWxhdGVkIHRvIGFzc2VzcyB0aGUgZWNvbm9taWMgZmVhc2liaWxpdHkgb2YgIG5hbm9idWJibGUsIHdoaWxlIGNvbW11bml0eSBpbnRlcnZpZXdzIHdlcmUgdXNlZCBmb3IgdGhlIHNvY2lhbCBzZWN0b3JzLiAgQmFzZWQgb24gdGhlIHJlc3VsdHMsIGVudmlyb25tZW50YWwgd2FzdGVzIG9mIHNocmltcCBmYXJtaW5nIGluIGZvcm0gb2YgIHRvdGFsIGFtbW9uaWEgbml0cm9nZW4gKFRBTikgd2VyZSByZWR1Y2VkIGJ5IDklIGZyb20gMi41OCBtZy9sICg5NSVDSTogIDAsOTEsIDQsMjUpIGluIGNvbnRyb2wgY29tcGFyZWQgdG8gMi4zNSBtZy9sICg5NSVDSTogMCw4NiwgMyw4NCkgaW4gIHRyZWF0bWVudC4gRnVydGhlcm1vcmUsIHRoZSByZXZlbnVlcyBmcm9tIHBvc3QgbGFydmEgc2FsZXMgZm9yIGZpdmUgeWVhcnMgYW5kICBuYW5vYnViYmxlIGludmVzdG1lbnQgY29zdHMgc2hvd2VkIHRoYXQgdGhlIGVzdGltYXRlZCBOUFYgd2FzIElEUiAgNjQsODI0LDM3NCB3aXRoIGEgcGF5YmFjayBwZXJpb2Qgb2YgMS43IHllYXJzLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjQ1LjQ4MzI4NzgxMTI3OTMsImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2LjgwMDU2NzYyNjk1MzEyNSwiaGVpZ2h0IjoxLjc0OTIzODYxMDI2NzYzOTJ9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo0Ny4xNjcxNzE0NzgyNzE0ODQsImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2Ljc3NTAzOTY3Mjg1MTU2LCJoZWlnaHQiOjEuNDA2Nzg1OTY0OTY1ODIwM319LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjQ4LjYyMzgwOTgxNDQ1MzEyNSwibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuNzcxMzAxMjY5NTMxMjUsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NTAuMDgwNDQ4MTUwNjM0NzY2LCJsZWZ0IjoyMS44Mjc3NzAyMzMxNTQyOTcsIndpZHRoIjo1Ni44MjAyOTcyNDEyMTA5NCwiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo1MS41MzcwOTAzMDE1MTM2NywibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuODI2NTMwNDU2NTQyOTcsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NTMuMDEwNjY1ODkzNTU0NjksImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2Ljc2NTcyNDE4MjEyODkwNiwiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo1NC40NjczMDQyMjk3MzYzMywibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuNzkzNzEyNjE1OTY2OCwiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo1NS45MjM5NDYzODA2MTUyMzQsImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2Ljc5NTYxNjE0OTkwMjM0NCwiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo1Ny4zODA1ODg1MzE0OTQxNCwibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuNzc2OTI3OTQ3OTk4MDUsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NTguODU0MTY0MTIzNTM1MTU2LCJsZWZ0IjoyMS44Mjc3NzAyMzMxNTQyOTcsIndpZHRoIjo1Ni43ODA2NjYzNTEzMTgzNiwiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2MC4zMTA3OTg2NDUwMTk1MywibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuODMzMzc0MDIzNDM3NSwiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2MS43Njc0NDA3OTU4OTg0NCwibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuNzQ3MDA5Mjc3MzQzNzUsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NjMuMjI4MzEzNDQ2MDQ0OTIsImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2Ljc4ODEyNzg5OTE2OTkyLCJoZWlnaHQiOjEuNDA2Nzg1OTY0OTY1ODIwM319LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjY0LjcwMTg4OTAzODA4NTk0LCJsZWZ0IjoyMS44Mjc3NzAyMzMxNTQyOTcsIndpZHRoIjo1Ni43ODYyNjI1MTIyMDcwMywiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2Ni4xNTg1MzExODg5NjQ4NCwibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuODE1MzE5MDYxMjc5MywiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI1MDA1In19XQ%3D%3D. Furthermore, the negatively charged nanobubble surfaces promote adhesion of beneficial bacteria (e.g., Bacillus spp.), stimulating probiotic proliferation that aids nutrient cycling and reduces uneaten feed residues (Jia et al., 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjMwLCJzdWJ0eXBlIjp7fSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTWluZ3lpIiwiZmFtaWx5IjoiSmlhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik11aGFtbWFkIFVzbWFuIiwiZmFtaWx5IjoiRmFyaWQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSmVoYWQgQS4iLCJmYW1pbHkiOiJLaGFycmF6In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hbGxhcGFuZW5pIE1hbm9qIiwiZmFtaWx5IjoiS3VtYXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2hhdWhyYXQgUy4iLCJmYW1pbHkiOiJDaG9wcmEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW0iLCJmYW1pbHkiOiJKYW5nIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlkuTS4gSm9obiIsImZhbWlseSI6IkNoZXcifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2FtaXIgS3VtYXIiLCJmYW1pbHkiOiJLaGFuYWwifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiR3VhbmdoYW8iLCJmYW1pbHkiOiJDaGVuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFsaWNpYSBLeW91bmdqaW4iLCJmYW1pbHkiOiJBbiJ9fV0sImlkcyI6eyJpc3NuIjpbIjAwNDMtMTM1NCIsIjE4NzktMjQ0OCJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMTAxNi9qLndhdHJlcy4yMDIzLjEyMDYxMyIsInBtaWQiOiIzNzczODk0MCIsIm9wZW5hbGV4IjoiVzQzODY2MTg0NTkifSwidGl0bGUiOiJOYW5vYnViYmxlcyBpbiB3YXRlciBhbmQgd2FzdGV3YXRlciB0cmVhdG1lbnQgc3lzdGVtczogU21hbGwgYnViYmxlcyBtYWtpbmcgYmlnIGRpZmZlcmVuY2UiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAxNi9qLndhdHJlcy4yMDIzLjEyMDYxMyIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9wdXJlaG9zdC5iYXRoLmFjLnVrL3dzL2ZpbGVzLzI5NjM0MzkwMS9OYW5vYnViYmxlc19wcmVfcHJvb2YucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjMtMDktMTEiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA4IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJXYXRlciBSZXNlYXJjaCIsInZvbHVtZSI6MjQ1LCJpbXBhY3RGYWN0b3IiOiIxMy40NCJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjEyMDYxMywibGFzdFBhZ2UiOjEyMDYxM319LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJFbHNldmllciBCViJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MTExLCJjaXRlZEJ5Q291bnQiOjc1fX0sInF1b3RlIjoiU2V2ZXJhbCBzdHVkaWVzIGhhdmUgcHJvcG9zZWQgdGhhdCB0aGUgbmVnYXRpdmUgY2hhcmdlIG9mIE5CcyBhcmlzZXMgZHVlIHRvIHRoZWlyIHNtYWxsIHJhZGl1cyBhbmQgdGhlIGhpZ2ggcG9sYXJpdHkgb2YgaHlkcm94eWwgaW9ucywgd2hpY2ggbGVhZCB0byB0aGUgYWRzb3JwdGlvbiBvZiBkaXNzb2NpYXRlZCBoeWRyb3h5bCBpb25zIG9uIHRoZSBidWJibGUgc3VyZmFjZSwgd2hpbGUgaHlkcm9nZW4gaW9ucyBhcmUgZWFzaWx5IGh5ZHJhdGVkIGFuZCBwcmVmZXIgdG8gc3RheSBpbiB0aGUgYnVsayBsaXF1aWQuIEEgaGlnaGVyIGFic29sdXRlIHpldGEgcG90ZW50aWFsIGluZGljYXRlcyBoaWdoZXIgc3RhYmlsaXR5LCBhcyB0aGUgcmVwdWxzaXZlIGZvcmNlIGdlbmVyYXRlZCBieSBOQnPigJggc3VyZmFjZXMgcHJldmVudHMgdGhlbSBmcm9tIG1lcmdpbmcgd2l0aCBvbmUgYW5vdGhlciAoVXNoaWt1Ym8gZXQgYWwuLCAyMDEwKS4gMTEgU3VyZmFjZSBoeWRyb3Bob2JpY2l0eSBpcyBhbm90aGVyIGVzc2VudGlhbCBwcm9wZXJ0eSBvZiBOQnMgYXMgaXQgYWxsb3dzIE5CcyB0byBpbnRlcmFjdCB3aXRoIG90aGVyIHBhcnRpY2xlcyB0aHJvdWdoIGh5ZHJvcGhvYmljIGF0dHJhY3Rpb24uIEJOQnMgY2FuIGFkc29yYiBvbiBoeWRyb3Bob2JpYyBwYXJ0aWNsZXMgYW5kIGluY3JlYXNlIHRoZWlyIG1vYmlsaXR5IGluIHRoZSBsaXF1aWQsIHdoaWxlIGF0dGFjaGVkIE5CcyBjYW4gcHJvbW90ZSBwYXJ0aWNsZS1wYXJ0aWNsZSBvciBwYXJ0aWNsZS0gYnViYmxlIGludGVyYWN0aW9ucyB0aHJvdWdoIGNhcGlsbGFyeSBicmlkZ2luZyBlZmZlY3RzIHRoYXQgcHJvZHVjZSBhbiBhdHRyYWN0aXZlIGZvcmNlIGJldHdlZW4gdHdvIHN1cmZhY2VzIChIYW1wdG9uIGFuZCBOZ3V5ZW4sIDIwMTApLiAyLjEuMyBMb25nIHJlc2lkZW5jZSB0aW1lIGluIHdhdGVyOiBOQnPigJkgbG9uZy10ZXJtIHN0YWJpbGl0eSBBbW9uZyB0aGUgdmFyaW91cyBjaGFyYWN0ZXJpc3RpY3MgTkJzIHBvc3Nlc3MsIHRoZSBtb3N0IGFzdG9uaXNoaW5nIGlzIHRoZWlyIGxvbmdldml0eS4gVXBvbiBnZW5lcmF0aW9uLCBidWJibGVzIGdlbmVyYWxseSBtYWludGFpbiB0aGVpciBkaWFtZXRlcnMgZm9yIGEgY2VydGFpbiBhbW91bnQgb2YgdGltZSwgYWZ0ZXIgd2hpY2ggdGhlIGJ1YmJsZXMgYmVnaW4gdG8gZ3JvdyBhbmQgZXZlbnR1YWxseSBkaXNzb2x2ZSBpbiB0aGUgc29sdXRpb24uIEV4cGVyaW1lbnRhbCByZXN1bHRzIGhhdmUgY29uZmlybWVkIHRoYXQgTkJzIGFyZSBhYmxlIHRvIGV4aXN0IGluIHdhdGVyIGZyb20gZGF5cyB0byBtb250aHMuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxMSwiYmJveCI6eyJ0b3AiOjY4LjYzMTk2NTYzNzIwNzAzLCJsZWZ0Ijo5Ljc0MTE4MDQxOTkyMTg3NSwid2lkdGgiOjc3LjAxMDI1MzkwNjI1LCJoZWlnaHQiOjEuODQ5MjYzNjY4MDYwMzAyN319LHsicGFnZU51bWJlciI6MTEsImJib3giOnsidG9wIjo3MS44MjM4MjIwMjE0ODQzOCwibGVmdCI6OS43NDExODA0MTk5MjE4NzUsIndpZHRoIjo3Ny4wMDYzMDk1MDkyNzczNCwiaGVpZ2h0IjoxLjg0OTI2MzY2ODA2MDMwMjd9fSx7InBhZ2VOdW1iZXIiOjExLCJiYm94Ijp7InRvcCI6NzUuMDEyMjE0NjYwNjQ0NTMsImxlZnQiOjkuNzQxMTgwNDE5OTIxODc1LCJ3aWR0aCI6NzYuOTQ4Nzc2MjQ1MTE3MTksImhlaWdodCI6MS44NDkyNjM2NjgwNjAzMDI3fX0seyJwYWdlTnVtYmVyIjoxMSwiYmJveCI6eyJ0b3AiOjc4LjIwMDU5OTY3MDQxMDE2LCJsZWZ0Ijo5Ljc0MTE4MDQxOTkyMTg3NSwid2lkdGgiOjc3LjAwODI4NTUyMjQ2MDk0LCJoZWlnaHQiOjEuODQ5MjYzNjY4MDYwMzAyN319LHsicGFnZU51bWJlciI6MTEsImJib3giOnsidG9wIjo4MS4zODg5OTk5Mzg5NjQ4NCwibGVmdCI6OS43NDExODA0MTk5MjE4NzUsIndpZHRoIjo3Ny4wMTAyNDYyNzY4NTU0NywiaGVpZ2h0IjoxLjg0OTI2MzY2ODA2MDMwMjd9fSx7InBhZ2VOdW1iZXIiOjExLCJiYm94Ijp7InRvcCI6ODQuNTc2OTE5NTU1NjY0MDYsImxlZnQiOjkuNzQxMTgwNDE5OTIxODc1LCJ3aWR0aCI6MTkuNjE2NjI0ODMyMTUzMzIsImhlaWdodCI6MS44NDkyNjM2NjgwNjAzMDI3fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjg3LjQ4ODA1OTk5NzU1ODYsImxlZnQiOjQ3LjAxODI1MzMyNjQxNjAxNiwid2lkdGgiOjIuNDUyMTYxNzg4OTQwNDI5NywiaGVpZ2h0IjoxLjg0OTI2MzY2ODA2MDMwMjd9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MTEuNjQ4NzM4ODYxMDgzOTg0LCJsZWZ0Ijo5Ljc0MTE4MDQxOTkyMTg3NSwid2lkdGgiOjc2Ljk1MjcwNTM4MzMwMDc4LCJoZWlnaHQiOjEuODQ5MjY1NTc1NDA4OTM1NX19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoxNC44Mzk0MzQ2MjM3MTgyNjIsImxlZnQiOjkuNzQxMTgwNDE5OTIxODc1LCJ3aWR0aCI6NzYuOTQ4NzgzODc0NTExNzIsImhlaWdodCI6MS44NDkyNjU1NzU0MDg5MzU1fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjE4LjAyNzgzMzkzODU5ODYzMywibGVmdCI6OS43NDExODA0MTk5MjE4NzUsIndpZHRoIjo3Ni41MDYwNzI5OTgwNDY4OCwiaGVpZ2h0IjoxLjg0OTI2NTU3NTQwODkzNTV9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjEuMjE2MjE3MDQxMDE1NjI1LCJsZWZ0Ijo5Ljc0MTE4MDQxOTkyMTg3NSwid2lkdGgiOjc2Ljk4NTczMzAzMjIyNjU2LCJoZWlnaHQiOjEuODQ5MjY1NTc1NDA4OTM1NX19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyNC40MDQ2MDAxNDM0MzI2MTcsImxlZnQiOjkuNzQxMTgwNDE5OTIxODc1LCJ3aWR0aCI6MzUuMzc0MTk4OTEzNTc0MjIsImhlaWdodCI6MS44NDkyNjU1NzU0MDg5MzU1fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjMyLjIxMTMxMTM0MDMzMjAzLCJsZWZ0Ijo5Ljc0MTE4MDQxOTkyMTg3NSwid2lkdGgiOjUyLjQ2NDEyMjc3MjIxNjgsImhlaWdodCI6MS45MTk5NjUyNjcxODEzOTY1fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM2LjA1MjYyNzU2MzQ3NjU2LCJsZWZ0Ijo5Ljc0MTE4MDQxOTkyMTg3NSwid2lkdGgiOjc3LjAwODI4NTUyMjQ2MDk0LCJoZWlnaHQiOjEuODQ5MjY3NDgyNzU3NTY4NH19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozOS4yNDEwMTYzODc5Mzk0NSwibGVmdCI6OS43NDExODA0MTk5MjE4NzUsIndpZHRoIjo3Ny4wMDgyODU1MjI0NjA5NCwiaGVpZ2h0IjoxLjg0OTI2NzQ4Mjc1NzU2ODR9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDIuNDI5NDA5MDI3MDk5NjEsImxlZnQiOjkuNzQxMTgwNDE5OTIxODc1LCJ3aWR0aCI6NzcuMDAyNDAzMjU5Mjc3MzQsImhlaWdodCI6MS44NDkyNjc0ODI3NTc1Njg0fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQ1LjYxNzc5NDAzNjg2NTIzNCwibGVmdCI6OS43NDExODA0MTk5MjE4NzUsIndpZHRoIjo3Ny4wMTAyNDYyNzY4NTU0NywiaGVpZ2h0IjoxLjg0OTI2NzQ4Mjc1NzU2ODR9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIxMjA2MjQifX1d.

Nanobubbles also foster symbiotic microbial communities that indirectly bolster organismal growth by refining water quality parameters essential for development. Enhanced DO curbs anaerobic pathogens while favoring nitrifying bacteria, leading to 9% reductions in total ammonia nitrogen and stabilized pH in shrimp ponds (Susanti et al., 2021)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkxpbHkiLCJmYW1pbHkiOiJTdXNhbnRpIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlN1eXVkIFdhcm5vIiwiZmFtaWx5IjoiVXRvbW8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTm92ZXJpdGEgRGlhbiIsImZhbWlseSI6IlRha2FyaW5hIn19XSwiaWRzIjp7Imlzc24iOlsiMjExNy00NDU4IiwiMjI3My0xNzA5Il0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDUxL2Jpb2NvbmYvMjAyMTMzMDUwMDUiLCJtYWciOiIzMTkzNTYzNzAzIiwib3BlbmFsZXgiOiJXMzE5MzU2MzcwMyJ9LCJ0aXRsZSI6IlN1c3RhaW5hYmlsaXR5IGFuZCBmZWFzaWJpbGl0eSBhc3Nlc3NtZW50cyBvZiBuYW5vYnViYmxlIGFlcmF0aW9uIHRlY2hub2xvZ3kgaW4gZWNvbm9taWMtc29jaW8gZW52aXJvbm1lbnQgb2YgUGVuYWV1cyB2YW5uYW1laSBzaHJpbXAgZmFybWluZyIsImFic3RyYWN0IjoiUGVuYWV1cyB2YW5uYW1laSBzaHJpbXAgZmFybWluZyBpbiBJbmRvbmVzaWEgaXMgZmFjZWQgd2l0aCBzZXZlcmFsIGNoYWxsZW5nZXMgaW4gdGhlIGVudmlyb25tZW50YWwsIGVjb25vbWljLCBhbmQgc29jaWFsIGFzcGVjdHMuIFRoZXJlZm9yZSwgdGhpcyBzdHVkeSBhaW1zIHRvIGFzc2VzcyB0aGUgYmVuZWZpdHMgb2Ygbm92ZWwgbmFub2J1YmJsZSBhZXJhdGlvbiBzeXN0ZW1zIGZvciB0aGUgZWNvc3lzdGVtLCBidXNpbmVzc2VzLCBhbmQgY29tbXVuaXRpZXMuIFRoaXMgaXMgYW4gZXhwZXJpbWVudGFsIHN0dWR5IGNvbmR1Y3RlZCBieSBjb21wYXJpbmcgUC4gdmFubmFtZWkgcG9zdCBsYXJ2YSAxMCByZWFyZWQgYXQgYSBkZW5zaXR5IG9mIDIwMDAgaW5kcy4vTCBpbiBhIHBvbmQgdHJlYXRlZCB3aXRoIG5hbm9idWJibGUgYW5kIHdpdGhvdXQgdHJlYXRtZW50cyAoY29udHJvbCkuIEZ1cnRoZXJtb3JlLCB0aGUgTmV0IFByZXNlbnQgVmFsdWUgKE5QVikgYW5kIHBheWJhY2sgcGVyaW9kIHdlcmUgY2FsY3VsYXRlZCB0byBhc3Nlc3MgdGhlIGVjb25vbWljIGZlYXNpYmlsaXR5IG9mIG5hbm9idWJibGUsIHdoaWxlIGNvbW11bml0eSBpbnRlcnZpZXdzIHdlcmUgdXNlZCBmb3IgdGhlIHNvY2lhbCBzZWN0b3JzLiBCYXNlZCBvbiB0aGUgcmVzdWx0cywgZW52aXJvbm1lbnRhbCB3YXN0ZXMgb2Ygc2hyaW1wIGZhcm1pbmcgaW4gZm9ybSBvZiB0b3RhbCBhbW1vbmlhIG5pdHJvZ2VuIChUQU4pIHdlcmUgcmVkdWNlZCBieSA5JSBmcm9tIDIuNTggbWcvbCAoOTUlQ0k6IDAsOTEsIDQsMjUpIGluIGNvbnRyb2wgY29tcGFyZWQgdG8gMi4zNSBtZy9sICg5NSVDSTogMCw4NiwgMyw4NCkgaW4gdHJlYXRtZW50LiBGdXJ0aGVybW9yZSwgdGhlIHJldmVudWVzIGZyb20gcG9zdCBsYXJ2YSBzYWxlcyBmb3IgZml2ZSB5ZWFycyBhbmQgbmFub2J1YmJsZSBpbnZlc3RtZW50IGNvc3RzIHNob3dlZCB0aGF0IHRoZSBlc3RpbWF0ZWQgTlBWIHdhcyBJRFIgNjQsODI0LDM3NCB3aXRoIGEgcGF5YmFjayBwZXJpb2Qgb2YgMS43IHllYXJzLiBUaGUgaW50ZXJ2aWV3cyBvbiB0cmFkaXRpb25hbCBzaHJpbXAgZmFybWluZyBzaG93ZWQgdGhhdCA2MS4xJSB0byA3Mi4yJSBvZiBjb21tdW5pdHkgbWVtYmVycyBhZ3JlZWQgb24gdGhlIHVzZSBvZiBuYW5vYnViYmxlIGFlcmF0aW9uIHRvIHN1cHBvcnQgbGl2ZWxpaG9vZHMuIFRoZXJlZm9yZSwgdGhlIHVzZSBvZiBuYW5vYnViYmxlcyBpcyBmZWFzaWJsZSB0byBzdXBwb3J0IHN1c3RhaW5hYmxlIFAuIHZhbm5hbWVpIGZhcm1pbmcuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjEwNTEvYmlvY29uZi8yMDIxMzMwNTAwNSIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cuYmlvLWNvbmZlcmVuY2VzLm9yZy9hcnRpY2xlcy9iaW9jb25mL3BkZi8yMDIxLzA1L2Jpb2NvbmZfaWNhdmVzczIwMjFfMDUwMDUucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjEtMDEtMDEiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA5IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJCSU8gV2ViIG9mIENvbmZlcmVuY2VzIiwidm9sdW1lIjozMywiaW1wYWN0RmFjdG9yIjoiMC42MiJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjUwMDUsImxhc3RQYWdlIjo1MDA1fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkVEUCBTY2llbmNlcyJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MTEsImNpdGVkQnlDb3VudCI6NH19LCJxdW90ZSI6IkFic3RyYWN0LiBQZW5hZXVzIHZhbm5hbWVpIHNocmltcCBmYXJtaW5nIGluIEluZG9uZXNpYSBpcyBmYWNlZCB3aXRoIHNldmVyYWwgY2hhbGxlbmdlcyBpbiB0aGUgZW52aXJvbm1lbnRhbCwgZWNvbm9taWMsIGFuZCBzb2NpYWwgYXNwZWN0cy4gIFRoZXJlZm9yZSwgdGhpcyBzdHVkeSBhaW1zIHRvIGFzc2VzcyB0aGUgYmVuZWZpdHMgb2Ygbm92ZWwgbmFub2J1YmJsZSAgYWVyYXRpb24gc3lzdGVtcyBmb3IgdGhlIGVjb3N5c3RlbSwgYnVzaW5lc3NlcywgYW5kIGNvbW11bml0aWVzLiBUaGlzIGlzIGFuICBleHBlcmltZW50YWwgc3R1ZHkgY29uZHVjdGVkIGJ5IGNvbXBhcmluZyBQLiB2YW5uYW1laSBwb3N0IGxhcnZhIDEwICByZWFyZWQgYXQgYSBkZW5zaXR5IG9mIDIwMDAgaW5kcy4vTCBpbiBhIHBvbmQgdHJlYXRlZCB3aXRoIG5hbm9idWJibGUgYW5kICB3aXRob3V0IHRyZWF0bWVudHMgKGNvbnRyb2wpLiBGdXJ0aGVybW9yZSwgdGhlIE5ldCBQcmVzZW50IFZhbHVlIChOUFYpIGFuZCAgcGF5YmFjayBwZXJpb2Qgd2VyZSBjYWxjdWxhdGVkIHRvIGFzc2VzcyB0aGUgZWNvbm9taWMgZmVhc2liaWxpdHkgb2YgIG5hbm9idWJibGUsIHdoaWxlIGNvbW11bml0eSBpbnRlcnZpZXdzIHdlcmUgdXNlZCBmb3IgdGhlIHNvY2lhbCBzZWN0b3JzLiAgQmFzZWQgb24gdGhlIHJlc3VsdHMsIGVudmlyb25tZW50YWwgd2FzdGVzIG9mIHNocmltcCBmYXJtaW5nIGluIGZvcm0gb2YgIHRvdGFsIGFtbW9uaWEgbml0cm9nZW4gKFRBTikgd2VyZSByZWR1Y2VkIGJ5IDklIGZyb20gMi41OCBtZy9sICg5NSVDSTogIDAsOTEsIDQsMjUpIGluIGNvbnRyb2wgY29tcGFyZWQgdG8gMi4zNSBtZy9sICg5NSVDSTogMCw4NiwgMyw4NCkgaW4gIHRyZWF0bWVudC4gRnVydGhlcm1vcmUsIHRoZSByZXZlbnVlcyBmcm9tIHBvc3QgbGFydmEgc2FsZXMgZm9yIGZpdmUgeWVhcnMgYW5kICBuYW5vYnViYmxlIGludmVzdG1lbnQgY29zdHMgc2hvd2VkIHRoYXQgdGhlIGVzdGltYXRlZCBOUFYgd2FzIElEUiAgNjQsODI0LDM3NCB3aXRoIGEgcGF5YmFjayBwZXJpb2Qgb2YgMS43IHllYXJzLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjQ1LjQ4MzI4NzgxMTI3OTMsImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2LjgwMDU2NzYyNjk1MzEyNSwiaGVpZ2h0IjoxLjc0OTIzODYxMDI2NzYzOTJ9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo0Ny4xNjcxNzE0NzgyNzE0ODQsImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2Ljc3NTAzOTY3Mjg1MTU2LCJoZWlnaHQiOjEuNDA2Nzg1OTY0OTY1ODIwM319LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjQ4LjYyMzgwOTgxNDQ1MzEyNSwibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuNzcxMzAxMjY5NTMxMjUsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NTAuMDgwNDQ4MTUwNjM0NzY2LCJsZWZ0IjoyMS44Mjc3NzAyMzMxNTQyOTcsIndpZHRoIjo1Ni44MjAyOTcyNDEyMTA5NCwiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo1MS41MzcwOTAzMDE1MTM2NywibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuODI2NTMwNDU2NTQyOTcsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NTMuMDEwNjY1ODkzNTU0NjksImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2Ljc2NTcyNDE4MjEyODkwNiwiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo1NC40NjczMDQyMjk3MzYzMywibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuNzkzNzEyNjE1OTY2OCwiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo1NS45MjM5NDYzODA2MTUyMzQsImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2Ljc5NTYxNjE0OTkwMjM0NCwiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo1Ny4zODA1ODg1MzE0OTQxNCwibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuNzc2OTI3OTQ3OTk4MDUsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NTguODU0MTY0MTIzNTM1MTU2LCJsZWZ0IjoyMS44Mjc3NzAyMzMxNTQyOTcsIndpZHRoIjo1Ni43ODA2NjYzNTEzMTgzNiwiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2MC4zMTA3OTg2NDUwMTk1MywibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuODMzMzc0MDIzNDM3NSwiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2MS43Njc0NDA3OTU4OTg0NCwibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuNzQ3MDA5Mjc3MzQzNzUsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NjMuMjI4MzEzNDQ2MDQ0OTIsImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2Ljc4ODEyNzg5OTE2OTkyLCJoZWlnaHQiOjEuNDA2Nzg1OTY0OTY1ODIwM319LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjY0LjcwMTg4OTAzODA4NTk0LCJsZWZ0IjoyMS44Mjc3NzAyMzMxNTQyOTcsIndpZHRoIjo1Ni43ODYyNjI1MTIyMDcwMywiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2Ni4xNTg1MzExODg5NjQ4NCwibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuODE1MzE5MDYxMjc5MywiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI1MDA1In19XQ%3D%3D. This microenvironment supports phytoplankton and zooplankton blooms, serving as live feed that improves larval survival and juvenile vigor. Studies on perforated-plate nanobubble generators highlight how geometric optimizations (e.g., hole diameter and count) maximize oxygen transfer efficiency at neutral-alkaline pH, aligning with pond conditions to enhance OH- adsorption and microbial activity (Wu et al., 2021; Zhu et al., 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjMwLCJzdWJ0eXBlIjp7fSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSmlhamlhIiwiZmFtaWx5IjoiV3UifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiS2VqaWEiLCJmYW1pbHkiOiJaaGFuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJDaGVuZyIsImZhbWlseSI6IkNlbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJYaWFvZ2FuZyIsImZhbWlseSI6Ild1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlJ1eWluIiwiZmFtaWx5IjoiTWFvIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Illpbmd5aW5nIiwiZmFtaWx5IjoiWmhlbmcifX1dLCJpZHMiOnsiaXNzbiI6WyIyMTkxLTA4NTUiXSwiaXNibiI6W10sImRvaSI6IjEwLjExODYvczEzNTY4LTAyMS0wMTI1NC0wIiwibWFnIjoiMzE3NTY4MzU2MSIsInBtaWQiOiIzNDE4NDEzNyIsInBtY2lkIjoiODIzOTEwOSIsIm9wZW5hbGV4IjoiVzMxNzU2ODM1NjEifSwidGl0bGUiOiJSb2xlIG9mIGJ1bGsgbmFub2J1YmJsZXMgaW4gcmVtb3Zpbmcgb3JnYW5pYyBwb2xsdXRhbnRzIGluIHdhc3Rld2F0ZXIgdHJlYXRtZW50IiwiYWJzdHJhY3QiOiJBYnN0cmFjdCBUaGUgb2NjdXJyZW5jZSBvZiBhIHZhcmlldHkgb2Ygb3JnYW5pYyBwb2xsdXRhbnRzIGhhcyBjb21wbGljYXRlZCB3YXN0ZXdhdGVyIHRyZWF0bWVudDsgdGh1cywgdGhlIHNlYXJjaCBmb3Igc3VzdGFpbmFibGUgYW5kIGVmZmVjdGl2ZSB0cmVhdG1lbnQgdGVjaG5vbG9neSBoYXMgZHJhd24gc2lnbmlmaWNhbnQgYXR0ZW50aW9uLiBJbiByZWNlbnQgeWVhcnMsIGJ1bGsgbmFub2J1YmJsZXMsIHdoaWNoIGhhdmUgZXh0cmFvcmRpbmFyeSBwcm9wZXJ0aWVzIGRpZmZlcmluZyBmcm9tIHRob3NlIG9mIG1pY3JvYnViYmxlcywgaW5jbHVkaW5nIGhpZ2ggc3RhYmlsaXR5IGFuZCBsb25nIHJlc2lkZW5jZSB0aW1lcyBpbiB3YXRlciwgbGFyZ2Ugc3BlY2lmaWMgc3VyZmFjZSBhcmVhcywgaGlnaCBnYXMgdHJhbnNmZXIgZWZmaWNpZW5jeSBhbmQgaW50ZXJmYWNlIHBvdGVudGlhbCwgYW5kIHRoZSBjYXBhYmlsaXR5IHRvIGdlbmVyYXRlIGZyZWUgcmFkaWNhbHMsIGhhdmUgc2hvd24gYXR0cmFjdGl2ZSB0ZWNobm9sb2dpY2FsIGFkdmFudGFnZXMgYW5kIHByb21pc2luZyBhcHBsaWNhdGlvbiBwcm9zcGVjdHMgZm9yIHdhc3Rld2F0ZXIgdHJlYXRtZW50LiBJbiB0aGlzIHJldmlldywgdGhlIGJhc2ljIGNoYXJhY3RlcmlzdGljcyBvZiBidWxrIG5hbm9idWJibGVzIGFyZSBzdW1tYXJpemVkIGluIGRldGFpbCwgYW5kIHJlY2VudCBmaW5kaW5ncyByZWxhdGVkIHRvIHRoZWlyIGltcGxlbWVudGF0aW9uIHBhdGh3YXlzIGFuZCBtZWNoYW5pc21zIGluIG9yZ2FuaWMgd2FzdGV3YXRlciB0cmVhdG1lbnQgYXJlIHN5c3RlbWF0aWNhbGx5IGRpc2N1c3NlZCwgd2hpY2ggaW5jbHVkZXMgaW1wcm92aW5nIHRoZSBhaXIgZmxvdGF0aW9uIHByb2Nlc3MsIGluY3JlYXNpbmcgd2F0ZXIgYWVyYXRpb24gdG8gcHJvbW90ZSBhZXJvYmljIGJpb2xvZ2ljYWwgdGVjaG5vbG9naWVzIGluY2x1ZGluZyBiaW9sb2dpY2FsIGFjdGl2YXRlZCBjYXJib24sIGFjdGl2YXRlZCBzbHVkZ2UsIGFuZCBtZW1icmFuZSBiaW9yZWFjdG9ycywgYW5kIGdlbmVyYXRpbmcgYWN0aXZlIGZyZWUgcmFkaWNhbHMgdGhhdCBveGlkaXNlIG9yZ2FuaWMgY29tcG91bmRzLiBGaW5hbGx5LCB0aGUgY3VycmVudCB0ZWNobm9sb2dpY2FsIGRpZmZpY3VsdGllcyBvZiBidWxrIG5hbm9idWJibGVzIGFyZSBhbmFseXNlZCwgYW5kIGZ1dHVyZSBmb2N1cyBhcmVhcyBmb3IgcmVzZWFyY2ggb24gYnVsayBuYW5vYnViYmxlIHRlY2hub2xvZ3kgYXJlIGFsc28gcHJvcG9zZWQuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjExODYvczEzNTY4LTAyMS0wMTI1NC0wIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2FtYi1leHByZXNzLnNwcmluZ2Vyb3Blbi5jb20vY291bnRlci9wZGYvMTAuMTE4Ni9zMTM1NjgtMDIxLTAxMjU0LTAiLCJpc3N1ZWREYXRlIjoiMjAyMS0wNi0yOCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkFNQiBFeHByZXNzIiwiaXNzdWUiOjEsInZvbHVtZSI6MTEsImltcGFjdEZhY3RvciI6IjQuMjYifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiU3ByaW5nZXIgTmF0dXJlIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo3NiwiY2l0ZWRCeUNvdW50Ijo3NX19LCJxdW90ZSI6IlRoZSBtb2RlbCBzdWNjZXNzZnVsbHkgZWx1Y2lkYXRlcyByZXB1bHNpdmUgZm9yY2VzICBiZXR3ZWVuIE9I4oiSIG9uIHRoZSBzdXJmYWNlIGFyZSBzdWZmaWNpZW50IHRvIHVsdGktbWF0ZWx5IGJhbGFuY2UgdGhlIHN1cmZhY2UgdGVuc2lvbiBmb3JjZXMgYXQgbmFub2J1YmJsZSAgc2l6ZXMsIHRodXMsIG1ha2luZyBuYW5vYnViYmxlcyBleGhpYml0IGdyZWF0IHN0YWJpbC1pdHkuIEFzIHNob3duIGluIHByZXZpb3VzIGxpdGVyYXR1cmVzLCB6ZXRhIHBvdGVudGlhbCB2YWwtdWVzIGNoYW5nZSB3aXRoIHRoZSBwSCBvZiB0aGUgbWVkaXVtIHZhcnksIGV4aGliaXRpbmcgIGEgdGVuZGVuY3kgdGhhdCB0aGUgbmVnYXRpdml0eSBvZiB6ZXRhIHBvdGVudGlhbCBpbmNyZWFzZXMgIHdpdGggc29sdXRpb24gcEggaW5jcmVhc2Ugd2hpY2ggcmVzdWx0cyBpbiBncmVhdGVyIGJ1Yi1ibGUgc3VyZmFjZSBjaGFyZ2UgYW5kIG1ha2UgYnViYmxlcyBtb3JlIHN0YWJsZSBkdWUgdG8gIGVsZWN0cm9zdGF0aWMgcmVwdWxzaW9uIChNZWVnb2RhIGV0IGFsLiAyMDE4KS4gTmFub2J1Yi1ibGVzIGdlbmVyYWxseSBzaG93IGhpZ2ggbmVnYXRpdmUgemV0YSBwb3RlbnRpYWwgb3dpbmcgdG8gIHRoZSBleGNlc3Mgb2YgaHlkcm94aWRlIGlvbnMgKE9I4oiSKSByZWxhdGl2ZSB0byBoeWRyb2dlbiAgaW9ucyAoSCspIGF0IHRoZSBnYXPigJN3YXRlciBpbnRlcmZhY2Ugb2YgYnViYmxlcyB1bmRlciAgd2lkZSByYW5nZSBvZiBzb2x1dGlvbiBwSCwgYnV0IG5hbm9idWJibGVzIGZvcm1lZCBpbiAgYWNpZGljIGVudmlyb25tZW50cyBhbHdheXMgc2hvdyBsb3dlciBzdGFiaWxpdHkgY29tLXBhcmVkIHdpdGggaW4gbmV1dHJhbCBvciBhbGthbGluZSBjb25kaXRpb25zIGJlY2F1c2UgYSAgbGFjayBvZiBPSOKIkiBhcmUgcmVxdWlyZWQgdG8gc3RhYmlsaXplIHRoZSBlbGVjdHJpYyBkb3VibGUgIGxheWVyIChOaXJtYWxrYXIgZXQgYWwuIDIwMTgpLCB0aGUgemV0YSBwb3RlbnRpYWwgb2YgbmFuby1idWJibGVzIGdlbmVyYXRlZCBpbiBzdHJvbmcgYWNpZGljIG1lZGl1bSBiZWxvdyBwSCAzICB0ZW5kcyB0byBiZSBwb3NpdGl2ZSBhbmQgbG93IHdoaWNoIGNhdXNlcyBoaWdoZXIgcG9zc2ktYmlsaXR5IGZvciBidWJibGUgY29hbGVzY2VuY2UgKFpoYW5nIGV0IGFsLiAyMDIwKS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjoxMy45NjgzMDU1ODc3Njg1NTUsImxlZnQiOjUyLjUzNDM0NzUzNDE3OTY5LCJ3aWR0aCI6MzguMzI2OTE1NzQwOTY2OCwiaGVpZ2h0IjoxLjgzNzY1NjYxNzE2NDYxMTh9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjoxNS40NzczMzQwMjI1MjE5NzMsImxlZnQiOjUxLjE5MDk3MTM3NDUxMTcyLCJ3aWR0aCI6MzkuMjg1MTYzODc5Mzk0NTMsImhlaWdodCI6MS44NDY1NjE1NTEwOTQwNTUyfX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MTcuMDAzNTYyOTI3MjQ2MDk0LCJsZWZ0Ijo1MS4xOTA0MzczMTY4OTQ1Mywid2lkdGgiOjM5LjY2OTEwNTUyOTc4NTE1NiwiaGVpZ2h0IjoxLjgzNzY1NjYxNzE2NDYxMTh9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjoxOC41MjAyNzcwMjMzMTU0MywibGVmdCI6NTEuMTkwNDM3MzE2ODk0NTMsIndpZHRoIjozOS4yODU2OTQxMjIzMTQ0NSwiaGVpZ2h0IjoxLjgzODI2NjI1MzQ3MTM3NDV9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjoyMC4wMzgyMTE4MjI1MDk3NjYsImxlZnQiOjUxLjE5MDQzNzMxNjg5NDUzLCJ3aWR0aCI6MzkuMjg1Njk0MTIyMzE0NDUsImhlaWdodCI6MS44Mzc2NTY2MTcxNjQ2MTE4fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MjEuNTU1NTM2MjcwMTQxNiwibGVmdCI6NTEuMTkwNDM3MzE2ODk0NTMsIndpZHRoIjozOS42NzUyMTY2NzQ4MDQ2OSwiaGVpZ2h0IjoxLjgzNzY1NjYxNzE2NDYxMTh9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjoyMy4wNzIyNDg0NTg4NjIzMDUsImxlZnQiOjUxLjE5MDQzNzMxNjg5NDUzLCJ3aWR0aCI6MzkuNjcwMzI2MjMyOTEwMTU2LCJoZWlnaHQiOjEuODM3NjU2NjE3MTY0NjExOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjI0LjU4ODk2MjU1NDkzMTY0LCJsZWZ0Ijo1MS4xOTA0MzczMTY4OTQ1Mywid2lkdGgiOjM5LjI4NTY5NDEyMjMxNDQ1LCJoZWlnaHQiOjEuODM4ODc1ODg5Nzc4MTM3Mn19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjI2LjEwNzUwNzcwNTY4ODQ3NywibGVmdCI6NTEuMTkwNDM3MzE2ODk0NTMsIndpZHRoIjozOS42NzIyNTI2NTUwMjkzLCJoZWlnaHQiOjEuODM3NjU2NjE3MTY0NjExOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjI3LjYyNDIyMTgwMTc1NzgxMiwibGVmdCI6NTEuMTkwNDM3MzE2ODk0NTMsIndpZHRoIjozOS4yODU2OTQxMjIzMTQ0NSwiaGVpZ2h0IjoxLjgzODI2NjI1MzQ3MTM3NDV9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjoyOS4xNDIxNTY2MDA5NTIxNSwibGVmdCI6NTEuMTkwNDM3MzE2ODk0NTMsIndpZHRoIjozOS42NzUyMzU3NDgyOTEwMTYsImhlaWdodCI6MS44Mzc2NTY2MTcxNjQ2MTE4fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MzAuNjUwNTczNzMwNDY4NzUsImxlZnQiOjUxLjE5MDQzNzMxNjg5NDUzLCJ3aWR0aCI6MzkuNjcxNzI2MjI2ODA2NjQsImhlaWdodCI6MS44NDY1NTg4MDkyODAzOTU1fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MzIuMTY3OTAwMDg1NDQ5MjIsImxlZnQiOjUxLjE4OTg5OTQ0NDU4MDA4LCJ3aWR0aCI6MzkuNjY5OTQwOTQ4NDg2MzMsImhlaWdodCI6MS44NDY1NTg4MDkyODAzOTU1fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MzMuNjkzNTE1Nzc3NTg3ODksImxlZnQiOjUxLjE5MDg2MDc0ODI5MTAxNiwid2lkdGgiOjM5LjY3NTcyMDIxNDg0Mzc1LCJoZWlnaHQiOjEuODM3NjU0NTkwNjA2Njg5NX19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjM1LjIxMDIzNTU5NTcwMzEyNSwibGVmdCI6NTEuMTkwODYwNzQ4MjkxMDE2LCJ3aWR0aCI6MzkuMjg1MjcwNjkwOTE3OTcsImhlaWdodCI6MS44Mzg4NzQxMDE2Mzg3OTR9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjozNi43Mjg3Nzg4MzkxMTEzMywibGVmdCI6NTEuMTkwNDM3MzE2ODk0NTMsIndpZHRoIjozOS42Njg5NzIwMTUzODA4NiwiaGVpZ2h0IjoxLjgzNzY1NDU5MDYwNjY4OTV9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjozOC4yMzcxNzg4MDI0OTAyMzQsImxlZnQiOjUxLjE5MDQzNzMxNjg5NDUzLCJ3aWR0aCI6MzkuNjY4ODA3OTgzMzk4NDQsImhlaWdodCI6MS44NDY1ODAwMjg1MzM5MzU1fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MzkuNzYyODIxMTk3NTA5NzY2LCJsZWZ0Ijo1MS4xODk5NTI4NTAzNDE4LCJ3aWR0aCI6MzkuMjg2MTgyNDAzNTY0NDUsImhlaWdodCI6MS44MzgyNjA1MzE0MjU0NzZ9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo0MS4yODA3NTAyNzQ2NTgyLCJsZWZ0Ijo1MS4xOTA0MzczMTY4OTQ1Mywid2lkdGgiOjM5LjY3MjczMzMwNjg4NDc2NiwiaGVpZ2h0IjoxLjgzNzY1NDU5MDYwNjY4OTV9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo0Mi43OTc0NzAwOTI3NzM0NCwibGVmdCI6NTEuMTkwNDM3MzE2ODk0NTMsIndpZHRoIjozOS4yODU2OTQxMjIzMTQ0NSwiaGVpZ2h0IjoxLjgzODI2MDUzMTQyNTQ3Nn19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjQ0LjMxNTM5OTE2OTkyMTg3NSwibGVmdCI6NTEuMTkwNDM3MzE2ODk0NTMsIndpZHRoIjozOS4yODU2OTQxMjIzMTQ0NSwiaGVpZ2h0IjoxLjgzNzY1NDU5MDYwNjY4OTV9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIzIn19LHsib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5pbmciLCJmYW1pbHkiOiJaaHUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTWluLVl1IiwiZmFtaWx5IjoiTGkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiS29oZWkiLCJmYW1pbHkiOiJTaGliYXRhIn19XSwiaWRzIjp7Imlzc24iOltdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMzM5MC9lbmdwcm9jMjAyMzA1NTAyMyIsIm9wZW5hbGV4IjoiVzQzODkxNDU1NzgiLCJleGEiOiJodHRwczovL3d3dy5tZHBpLmNvbS8yNjczLTQ1OTEvNTUvMS8yMyJ9LCJ0aXRsZSI6IlRoZSBEZXZlbG9wbWVudCBvZiBhIEhpZ2gtQ29uY2VudHJhdGlvbiBPeHlnZW5hdGVkIFdhdGVyIEdlbmVyYXRvciBCYXNlZCBvbiBOYW5vYnViYmxlcyBhbmQgSXRzIEFwcGxpY2F0aW9uIiwiYWJzdHJhY3QiOiJXYXRlciB3aXRoIGEgaGlnaCBjb25jZW50cmF0aW9uIG9mIG94eWdlbiBpcyBuZWVkZWQgZm9yIHRoZSBhcXVhY3VsdHVyZSBpbmR1c3RyeSBpbiBKYXBhbi4gSW4gdGhlIGN1cnJlbnQgc3R1ZHksIHRoZSBwcmVzc3VyaXplZCBkaXNzb2x1dGlvbiBtZXRob2Qgd2FzIGVtcGxveWVkIHRvIGdlbmVyYXRlIGhpZ2gtY29uY2VudHJhdGlvbiBveHlnZW5hdGVkIHdhdGVyIChIQ09XKSBieSBwcm9kdWNpbmcgb3h5Z2VuIG5hbm9idWJibGVzIGluIHRoZSB3YXRlci4gSW4gb3JkZXIgdG8gaW52ZXN0aWdhdGUgZmFjdG9ycyBzdWNoIGFzIHRlbXBlcmF0dXJlLCBnZW9tZXRyaWMgY29uZGl0aW9ucywgYW5kIHRoZWlyIGluZmx1ZW5jZSBvbiB0aGUgb3h5Z2VuIGNvbmNlbnRyYXRpb24sIGEgc3BlY2lhbCBuYW5vYnViYmxlIGdlbmVyYXRvciB3YXMgaW1wcm92ZWQgYnkgY2hhbmdpbmcgdGhlIG51bWJlciBhbmQgdGhlIGRpYW1ldGVyIG9mIHRoZSBob2xlcyBvZiB0aGUgcGVyZm9yYXRlZCBwbGF0ZSBpbiB0aGlzIHN0dWR5LiBUaGVuLCBhbiBleHBlcmltZW50YWwgc3lzdGVtIHdoZXJlIG94eWdlbiBhbmQgd2F0ZXIgd2VyZSBzZXBhcmF0ZWx5IGludHJvZHVjZWQgaW5zaWRlIHRoZSBwcm9wb3NlZCBuYW5vYnViYmxlIGdlbmVyYXRvciB3YXMgZGVzaWduZWQuIFRoZSBkaXNzb2x2ZWQgb3h5Z2VuIGNvbmNlbnRyYXRpb24gd2FzIG1lYXN1cmVkIHVuZGVyIGRpZmZlcmVudCBjb25kaXRpb25zLiBGaW5hbGx5LCB0aGUgcHJvZHVjZWQgSENPVyB3YXMgdXNlZCB0byBjdWx0aXZhdGUgYSBtaW5pLXN1bmZsb3dlci4gVGhyb3VnaCBhIHNlcmllcyBvZiBleHBlcmltZW50cywgaXQgd2FzIGZvdW5kIHRoYXQgd2l0aCB0aGUgaW1wcm92ZWQgcGVyZm9yYXRlZCBwbGF0ZSwgdGhlIGRpc3NvbHZlZCBveHlnZW4gY29uY2VudHJhdGlvbiB3YXMgaW5jcmVhc2VkIGFuZCB0aGUgbmFub2J1YmJsZSBnZW5lcmF0b3IgcmVhY2hlZCB0aGUgc2F0dXJhdGlvbiBzdGF0ZSBxdWlja2x5LCB3aGlsZSB0aGUgbWluaS1zdW5mbG93ZXIgY3VsdGl2YXRlZCB3aXRoIHRoZSBIQ09XIGFwcGVhcmVkIHRvIGdyb3cgbGFyZ2VyIHRoYW4gdGhhdCB3aXRoIHRhcCB3YXRlci4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMzM5MC9lbmdwcm9jMjAyMzA1NTAyMyIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cubWRwaS5jb20vMjY3My00NTkxLzU1LzEvMjMvcGRmP3ZlcnNpb249MTcwMTI0MzExOCIsImlzc3VlZERhdGUiOiIyMDIzLTExLTI5IiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInZvbHVtZSI6MzN9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjIzLCJsYXN0UGFnZSI6MjN9fSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjIsImNpdGVkQnlDb3VudCI6MX19LCJxdW90ZSI6IkFsdG1ldHJpY1tfc2hhcmVfIFNoYXJlXShodHRwczovL3d3dy5tZHBpLmNvbS8yNjczLTQ1OTEvNTUvMS8yMykgW19hbm5vdW5jZW1lbnRfIEhlbHBdKGh0dHBzOi8vd3d3Lm1kcGkuY29tLzI2NzMtNDU5MS81NS8xLzIzKSBbX2Zvcm1hdFxcX3F1b3RlXyBDaXRlXShqYXZhc2NyaXB0OnZvaWQoMCk7KSBbX3F1ZXN0aW9uXFxfYW5zd2VyXyBEaXNjdXNzIGluIFNjaVByb2ZpbGVzXShodHRwczovL3NjaXByb2ZpbGVzLmNvbS9kaXNjdXNzaW9uLWdyb3Vwcy9wdWJsaWMvMTAuMzM5MC9lbmdwcm9jMjAyMzA1NTAyMz91dG1fc291cmNlPW1wZGkuY29tJnV0bV9tZWRpdW09cHVibGljYXRpb24mdXRtX2NhbXBhaWduPWRpc2N1c3NfaW5fc2NpcHJvZmlsZXMpIFdhdGVyIHdpdGggYSBoaWdoIGNvbmNlbnRyYXRpb24gb2Ygb3h5Z2VuIGlzIG5lZWRlZCBmb3IgdGhlIGFxdWFjdWx0dXJlIGluZHVzdHJ5IGluIEphcGFuLiBJbiB0aGUgY3VycmVudCBzdHVkeSwgdGhlIHByZXNzdXJpemVkIGRpc3NvbHV0aW9uIG1ldGhvZCB3YXMgZW1wbG95ZWQgdG8gZ2VuZXJhdGUgaGlnaC1jb25jZW50cmF0aW9uIG94eWdlbmF0ZWQgd2F0ZXIgKEhDT1cpIGJ5IHByb2R1Y2luZyBveHlnZW4gbmFub2J1YmJsZXMgaW4gdGhlIHdhdGVyLiBJbiBvcmRlciB0byBpbnZlc3RpZ2F0ZSBmYWN0b3JzIHN1Y2ggYXMgdGVtcGVyYXR1cmUsIGdlb21ldHJpYyBjb25kaXRpb25zLCBhbmQgdGhlaXIgaW5mbHVlbmNlIG9uIHRoZSBveHlnZW4gY29uY2VudHJhdGlvbiwgYSBzcGVjaWFsIG5hbm9idWJibGUgZ2VuZXJhdG9yIHdhcyBpbXByb3ZlZCBieSBjaGFuZ2luZyB0aGUgbnVtYmVyIGFuZCB0aGUgZGlhbWV0ZXIgb2YgdGhlIGhvbGVzIG9mIHRoZSBwZXJmb3JhdGVkIHBsYXRlIGluIHRoaXMgc3R1ZHkuIn1d. Consequently, integrated systems yield shorter culture cycles and higher yields, with field trials reporting 30% productivity gains in whiteleg shrimp farming.

Economic analyses further validate nanobubbles' role in growth promotion, projecting positive net present values (NPV > IDR 64 million over five years) and payback periods under 2 years for P. vannamei operations (Susanti et al., 2021)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkxpbHkiLCJmYW1pbHkiOiJTdXNhbnRpIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlN1eXVkIFdhcm5vIiwiZmFtaWx5IjoiVXRvbW8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTm92ZXJpdGEgRGlhbiIsImZhbWlseSI6IlRha2FyaW5hIn19XSwiaWRzIjp7Imlzc24iOlsiMjExNy00NDU4IiwiMjI3My0xNzA5Il0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDUxL2Jpb2NvbmYvMjAyMTMzMDUwMDUiLCJtYWciOiIzMTkzNTYzNzAzIiwib3BlbmFsZXgiOiJXMzE5MzU2MzcwMyJ9LCJ0aXRsZSI6IlN1c3RhaW5hYmlsaXR5IGFuZCBmZWFzaWJpbGl0eSBhc3Nlc3NtZW50cyBvZiBuYW5vYnViYmxlIGFlcmF0aW9uIHRlY2hub2xvZ3kgaW4gZWNvbm9taWMtc29jaW8gZW52aXJvbm1lbnQgb2YgUGVuYWV1cyB2YW5uYW1laSBzaHJpbXAgZmFybWluZyIsImFic3RyYWN0IjoiUGVuYWV1cyB2YW5uYW1laSBzaHJpbXAgZmFybWluZyBpbiBJbmRvbmVzaWEgaXMgZmFjZWQgd2l0aCBzZXZlcmFsIGNoYWxsZW5nZXMgaW4gdGhlIGVudmlyb25tZW50YWwsIGVjb25vbWljLCBhbmQgc29jaWFsIGFzcGVjdHMuIFRoZXJlZm9yZSwgdGhpcyBzdHVkeSBhaW1zIHRvIGFzc2VzcyB0aGUgYmVuZWZpdHMgb2Ygbm92ZWwgbmFub2J1YmJsZSBhZXJhdGlvbiBzeXN0ZW1zIGZvciB0aGUgZWNvc3lzdGVtLCBidXNpbmVzc2VzLCBhbmQgY29tbXVuaXRpZXMuIFRoaXMgaXMgYW4gZXhwZXJpbWVudGFsIHN0dWR5IGNvbmR1Y3RlZCBieSBjb21wYXJpbmcgUC4gdmFubmFtZWkgcG9zdCBsYXJ2YSAxMCByZWFyZWQgYXQgYSBkZW5zaXR5IG9mIDIwMDAgaW5kcy4vTCBpbiBhIHBvbmQgdHJlYXRlZCB3aXRoIG5hbm9idWJibGUgYW5kIHdpdGhvdXQgdHJlYXRtZW50cyAoY29udHJvbCkuIEZ1cnRoZXJtb3JlLCB0aGUgTmV0IFByZXNlbnQgVmFsdWUgKE5QVikgYW5kIHBheWJhY2sgcGVyaW9kIHdlcmUgY2FsY3VsYXRlZCB0byBhc3Nlc3MgdGhlIGVjb25vbWljIGZlYXNpYmlsaXR5IG9mIG5hbm9idWJibGUsIHdoaWxlIGNvbW11bml0eSBpbnRlcnZpZXdzIHdlcmUgdXNlZCBmb3IgdGhlIHNvY2lhbCBzZWN0b3JzLiBCYXNlZCBvbiB0aGUgcmVzdWx0cywgZW52aXJvbm1lbnRhbCB3YXN0ZXMgb2Ygc2hyaW1wIGZhcm1pbmcgaW4gZm9ybSBvZiB0b3RhbCBhbW1vbmlhIG5pdHJvZ2VuIChUQU4pIHdlcmUgcmVkdWNlZCBieSA5JSBmcm9tIDIuNTggbWcvbCAoOTUlQ0k6IDAsOTEsIDQsMjUpIGluIGNvbnRyb2wgY29tcGFyZWQgdG8gMi4zNSBtZy9sICg5NSVDSTogMCw4NiwgMyw4NCkgaW4gdHJlYXRtZW50LiBGdXJ0aGVybW9yZSwgdGhlIHJldmVudWVzIGZyb20gcG9zdCBsYXJ2YSBzYWxlcyBmb3IgZml2ZSB5ZWFycyBhbmQgbmFub2J1YmJsZSBpbnZlc3RtZW50IGNvc3RzIHNob3dlZCB0aGF0IHRoZSBlc3RpbWF0ZWQgTlBWIHdhcyBJRFIgNjQsODI0LDM3NCB3aXRoIGEgcGF5YmFjayBwZXJpb2Qgb2YgMS43IHllYXJzLiBUaGUgaW50ZXJ2aWV3cyBvbiB0cmFkaXRpb25hbCBzaHJpbXAgZmFybWluZyBzaG93ZWQgdGhhdCA2MS4xJSB0byA3Mi4yJSBvZiBjb21tdW5pdHkgbWVtYmVycyBhZ3JlZWQgb24gdGhlIHVzZSBvZiBuYW5vYnViYmxlIGFlcmF0aW9uIHRvIHN1cHBvcnQgbGl2ZWxpaG9vZHMuIFRoZXJlZm9yZSwgdGhlIHVzZSBvZiBuYW5vYnViYmxlcyBpcyBmZWFzaWJsZSB0byBzdXBwb3J0IHN1c3RhaW5hYmxlIFAuIHZhbm5hbWVpIGZhcm1pbmcuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjEwNTEvYmlvY29uZi8yMDIxMzMwNTAwNSIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cuYmlvLWNvbmZlcmVuY2VzLm9yZy9hcnRpY2xlcy9iaW9jb25mL3BkZi8yMDIxLzA1L2Jpb2NvbmZfaWNhdmVzczIwMjFfMDUwMDUucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjEtMDEtMDEiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA5IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJCSU8gV2ViIG9mIENvbmZlcmVuY2VzIiwidm9sdW1lIjozMywiaW1wYWN0RmFjdG9yIjoiMC42MiJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjUwMDUsImxhc3RQYWdlIjo1MDA1fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkVEUCBTY2llbmNlcyJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MTEsImNpdGVkQnlDb3VudCI6NH19LCJxdW90ZSI6IkFic3RyYWN0LiBQZW5hZXVzIHZhbm5hbWVpIHNocmltcCBmYXJtaW5nIGluIEluZG9uZXNpYSBpcyBmYWNlZCB3aXRoIHNldmVyYWwgY2hhbGxlbmdlcyBpbiB0aGUgZW52aXJvbm1lbnRhbCwgZWNvbm9taWMsIGFuZCBzb2NpYWwgYXNwZWN0cy4gIFRoZXJlZm9yZSwgdGhpcyBzdHVkeSBhaW1zIHRvIGFzc2VzcyB0aGUgYmVuZWZpdHMgb2Ygbm92ZWwgbmFub2J1YmJsZSAgYWVyYXRpb24gc3lzdGVtcyBmb3IgdGhlIGVjb3N5c3RlbSwgYnVzaW5lc3NlcywgYW5kIGNvbW11bml0aWVzLiBUaGlzIGlzIGFuICBleHBlcmltZW50YWwgc3R1ZHkgY29uZHVjdGVkIGJ5IGNvbXBhcmluZyBQLiB2YW5uYW1laSBwb3N0IGxhcnZhIDEwICByZWFyZWQgYXQgYSBkZW5zaXR5IG9mIDIwMDAgaW5kcy4vTCBpbiBhIHBvbmQgdHJlYXRlZCB3aXRoIG5hbm9idWJibGUgYW5kICB3aXRob3V0IHRyZWF0bWVudHMgKGNvbnRyb2wpLiBGdXJ0aGVybW9yZSwgdGhlIE5ldCBQcmVzZW50IFZhbHVlIChOUFYpIGFuZCAgcGF5YmFjayBwZXJpb2Qgd2VyZSBjYWxjdWxhdGVkIHRvIGFzc2VzcyB0aGUgZWNvbm9taWMgZmVhc2liaWxpdHkgb2YgIG5hbm9idWJibGUsIHdoaWxlIGNvbW11bml0eSBpbnRlcnZpZXdzIHdlcmUgdXNlZCBmb3IgdGhlIHNvY2lhbCBzZWN0b3JzLiAgQmFzZWQgb24gdGhlIHJlc3VsdHMsIGVudmlyb25tZW50YWwgd2FzdGVzIG9mIHNocmltcCBmYXJtaW5nIGluIGZvcm0gb2YgIHRvdGFsIGFtbW9uaWEgbml0cm9nZW4gKFRBTikgd2VyZSByZWR1Y2VkIGJ5IDklIGZyb20gMi41OCBtZy9sICg5NSVDSTogIDAsOTEsIDQsMjUpIGluIGNvbnRyb2wgY29tcGFyZWQgdG8gMi4zNSBtZy9sICg5NSVDSTogMCw4NiwgMyw4NCkgaW4gIHRyZWF0bWVudC4gRnVydGhlcm1vcmUsIHRoZSByZXZlbnVlcyBmcm9tIHBvc3QgbGFydmEgc2FsZXMgZm9yIGZpdmUgeWVhcnMgYW5kICBuYW5vYnViYmxlIGludmVzdG1lbnQgY29zdHMgc2hvd2VkIHRoYXQgdGhlIGVzdGltYXRlZCBOUFYgd2FzIElEUiAgNjQsODI0LDM3NCB3aXRoIGEgcGF5YmFjayBwZXJpb2Qgb2YgMS43IHllYXJzLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjQ1LjQ4MzI4NzgxMTI3OTMsImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2LjgwMDU2NzYyNjk1MzEyNSwiaGVpZ2h0IjoxLjc0OTIzODYxMDI2NzYzOTJ9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo0Ny4xNjcxNzE0NzgyNzE0ODQsImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2Ljc3NTAzOTY3Mjg1MTU2LCJoZWlnaHQiOjEuNDA2Nzg1OTY0OTY1ODIwM319LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjQ4LjYyMzgwOTgxNDQ1MzEyNSwibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuNzcxMzAxMjY5NTMxMjUsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NTAuMDgwNDQ4MTUwNjM0NzY2LCJsZWZ0IjoyMS44Mjc3NzAyMzMxNTQyOTcsIndpZHRoIjo1Ni44MjAyOTcyNDEyMTA5NCwiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo1MS41MzcwOTAzMDE1MTM2NywibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuODI2NTMwNDU2NTQyOTcsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NTMuMDEwNjY1ODkzNTU0NjksImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2Ljc2NTcyNDE4MjEyODkwNiwiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo1NC40NjczMDQyMjk3MzYzMywibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuNzkzNzEyNjE1OTY2OCwiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo1NS45MjM5NDYzODA2MTUyMzQsImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2Ljc5NTYxNjE0OTkwMjM0NCwiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo1Ny4zODA1ODg1MzE0OTQxNCwibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuNzc2OTI3OTQ3OTk4MDUsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NTguODU0MTY0MTIzNTM1MTU2LCJsZWZ0IjoyMS44Mjc3NzAyMzMxNTQyOTcsIndpZHRoIjo1Ni43ODA2NjYzNTEzMTgzNiwiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2MC4zMTA3OTg2NDUwMTk1MywibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuODMzMzc0MDIzNDM3NSwiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2MS43Njc0NDA3OTU4OTg0NCwibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuNzQ3MDA5Mjc3MzQzNzUsImhlaWdodCI6MS40MDY3ODU5NjQ5NjU4MjAzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NjMuMjI4MzEzNDQ2MDQ0OTIsImxlZnQiOjIxLjgyNzc3MDIzMzE1NDI5Nywid2lkdGgiOjU2Ljc4ODEyNzg5OTE2OTkyLCJoZWlnaHQiOjEuNDA2Nzg1OTY0OTY1ODIwM319LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjY0LjcwMTg4OTAzODA4NTk0LCJsZWZ0IjoyMS44Mjc3NzAyMzMxNTQyOTcsIndpZHRoIjo1Ni43ODYyNjI1MTIyMDcwMywiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2Ni4xNTg1MzExODg5NjQ4NCwibGVmdCI6MjEuODI3NzcwMjMzMTU0Mjk3LCJ3aWR0aCI6NTYuODE1MzE5MDYxMjc5MywiaGVpZ2h0IjoxLjQwNjc4NTk2NDk2NTgyMDN9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI1MDA1In19XQ%3D%3D. Scalable generators like the aQua+75MO system (168.9 nm average size, 1.04 times 10^9 particles/mL) exemplify practical deployment, minimizing energy inputs while amplifying outputs (Nghĩa et al., 2022)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5ndXnhu4VuIEjhu691IiwiZmFtaWx5IjoiTmdoxKlhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5ndXllbiBUaGkiLCJmYW1pbHkiOiJOZ3V5ZW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUGhhbiBUcuG7jW5nIiwiZmFtaWx5IjoiQsOsbmgifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTGUgVGhpIiwiZmFtaWx5IjoiTWF5In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRvbmcgVHJhbiIsImZhbWlseSI6Ikh1eSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQaOG6oW0gVGjDoWkiLCJmYW1pbHkiOiJHaWFuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTb3BoaWUiLCJmYW1pbHkiOiJTdOKAkEhpbGFpcmUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUGhhbiBUaGkiLCJmYW1pbHkiOiJWYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMjM0LTE3NDkiLCIyMjM0LTE3NTciXSwiaXNibiI6W10sImRvaSI6IjEwLjQ3ODUzL2Zhcy4yMDIyLmUzOSIsIm9wZW5hbGV4IjoiVzQyOTQ1OTc1NTQifSwidGl0bGUiOiJFZmZlY3Qgb2YgbmFub2J1YmJsZXMgKG94eWdlbiwgb3pvbmUpIG9uIHRoZSBQYWNpZmljIHdoaXRlIHNocmltcCAoUGVuYWV1cyB2YW5uYW1laSksIFZpYnJpbyBwYXJhaGFlbW9seXRpY3VzIGFuZCB3YXRlciBxdWFsaXR5IHVuZGVyIGxhYiBjb25kaXRpb25zIiwiYWJzdHJhY3QiOiJUaGlzIHN0dWR5IGFzc2Vzc2VkIHRoZSBlZmZlY3RzIG9mIG94eWdlbiBhbmQgb3pvbmUgbmFub2J1YmJsZXMgb24gZ2lsbCBtb3JwaG9sb2d5LCB3ZWlnaHQgZ2FpbiwgYW5kIG1vcnRhbGl0eSBvZiBQYWNpZmljIHdoaXRlIHNocmltcCAoUGVuYWV1cyB2YW5uYW1laSksIGFzIHdlbGwgYXMgdGhlIGxldmVsIG9mIFZpYnJpbyBwYXJhaGFlbW9seXRpY3VzIGFuZCB3YXRlciBxdWFsaXR5IG9mIHNocmltcCBjdWx0dXJlIHRhbmtzIHVuZGVyIGxhYiBjb25kaXRpb25zLiBUd28gZXhwZXJpbWVudHMgd2VyZSBjYXJyaWVkIG91dCB3aXRoIG94eWdlbiBtYWNyb2J1YmJsZSwgb3pvbmUgbWFjcm9idWJibGUsIG94eWdlbiBuYW5vYnViYmxlLCBvem9uZSBuYW5vYnViYmxlLCBhbmQgY29udHJvbCB0cmVhdG1lbnRzIChhaXItc3RvbmUgbWFjcm9idWJibGUpLiBFeHBlcmltZW50cyB3ZXJlIGRvbmUgaW4gdHJpcGxpY2F0ZSBpbiAxMDAgTCB0YW5rcyB3aXRoIDE14oCwIHNhbGluZSB3YXRlciwgYW5kIDIwIHNocmltcCBwZXIgdGFuay4gVGFua3MgaW4gRXhwZXJpbWVudCAxIHdlcmUgbm90IGlub2N1bGF0ZWQgd2l0aCBiYWN0ZXJpYTsgdGFua3MgaW4gRXhwZXJpbWVudCAyIHdlcmUgaW5vY3VsYXRlZCB3aXRoIFYuIHBhcmFoYWVtb2x5dGljdXMgYXQgYSBjb25jZW50cmF0aW9uIG9mIDEwNiBDRlUvbUwuIFRoZSByZXN1bHRzIHJldmVhbGVkIHRoYXQgc2hvcnQgdHJlYXRtZW50cyB3aXRoIG96b25lIG5hbm9idWJibGVzIGhhZCBtaW5pbWFsIGltcGFjdCBvbiBzaHJpbXAgZ2lsbHMsIG1vcnRhbGl0eSwgYW5kIGdyb3d0aCByYXRlcywgcmVkdWNlZCBWLiBwYXJhaGFlbW9seXRpY3VzIGNvbmNlbnRyYXRpb24gaW4gd2F0ZXIgY29tcGFyZWQgdG8gdGhlIG90aGVyIGdyb3VwcywgYW5kIGltcHJvdmUgd2F0ZXIgcXVhbGl0eS4gVGhlc2UgbGFib3JhdG9yeSByZXN1bHRzIGluZGljYXRlIHRoYXQgb3pvbmUgbmFub2J1YmJsZSB0cmVhdG1lbnQgbWF5IGJlIHVzZWZ1bCBmb3IgY29udHJvbGxpbmcgVi4gcGFyYWhhZW1vbHl0aWN1cy4gTW9yZSB3b3JrIGlzIG5lZWRlZCB0byBmaW5kIHRoZSBiZXN0IHByb3RvY29sIHRvIGFwcGx5IHRoZSB0ZWNobm9sb2d5IG9uIGEgY29tbWVyY2lhbCBzY2FsZS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuNDc4NTMvZmFzLjIwMjIuZTM5IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5lLWZhcy5vcmcvZG93bmxvYWQvZG93bmxvYWRfcGRmP3BpZD1mYXMtMjUtOC00MjkiLCJpc3N1ZWREYXRlIjoiMjAyMi0wOC0wMSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZpc2hlcmllcyBhbmQgYXF1YXRpYyBzY2llbmNlcyIsImlzc3VlIjo4LCJ2b2x1bWUiOjI1LCJpbXBhY3RGYWN0b3IiOiIxLjM0In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6NDI5LCJsYXN0UGFnZSI6NDQwfX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlNwcmluZ2VyIFNjaWVuY2UrQnVzaW5lc3MgTWVkaWEifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjMwLCJjaXRlZEJ5Q291bnQiOjEzfX0sInF1b3RlIjoiQSBuYW5vYnViYmxlIGdlbmVyYXRvciBtb2RlbCBhUXVhKzc1TU8gKEFxdWFQcm8gU29sdXRpb25zLCBTaW5nYXBvcmUsIFNpbmdhcG9yZSkgd2l0aCBhIDEgSFAgcG93ZXIgKDAuNzUga1cpIGh0dHBzOi8vZG9pLm9yZy8xMC40Nzg1My9GQVMuMjAyMi5lMzkgaHR0cHM6Ly93d3cuZS1mYXMub3JnIHwgNDMxIE5ndXllbiBIdXUgTmdoaWEsIGV0IGFsLiBGaXNoZXJpZXMgYW5kIEFxdWF0aWMgU2NpZW5jZXMgbW90b3IgYW5kIGEgd2F0ZXIgZmxvdyBvdXRwdXQgcmF0ZSBvZiAxLDAwMCBML2ggd2FzIHVzZWQgdG8gZ2VuZXJhdGUgbmFub2J1YmJsZXMuIFRoZSBtYWNoaW5lIHByb2R1Y2VkIG5hbm9idWJibGVzICB3aXRoIGFuIGF2ZXJhZ2Ugc2l6ZSBvZiAxNjguOSDCsSA3My44IG5tLCBhbmQgYSBjb25jZW50cmF0aW9uIG9mICAxLjA0IMOXIDEwIDkgwrEgMi42IMOXIDEwIDggcGFydGljbGVzL21MIChBcXVhUHJvIFNvbHV0aW9ucywgMjAxOSkuIFRvIGNyZWF0ZSBveHlnZW4gbmFub2J1YmJsZXMsIHdlIHVzZWQgYW4gb3h5Z2VuIGNvbmNlbnRyYXRvciAobW9kZWwgWXV3ZWxsIDdGLTEwLCBZdXdlbGwsIERhbnlhbmcsIENoaW5hOyAxMCBML21pbiBtYXggIGZsb3dyYXRlLCDiiaUgOTUuNSUgb3h5Z2VuIGNvbmNlbnRyYXRpb24sIGZsb3cgcmF0ZSBhdCA0IEwvbWluKSAgcHJvdmlkZWQgb3h5Z2VuIGZvciBuYW5vYnViYmxlIGdlbmVyYXRvci4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo4Ni4yMjE0MDUwMjkyOTY4OCwibGVmdCI6NTQuMDQ3NTUwMjAxNDE2MDE2LCJ3aWR0aCI6MzguMjAxMjQ4MTY4OTQ1MzEsImhlaWdodCI6MS42OTk2MzEzMzMzNTExMzUzfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6ODcuOTg1Mjk4MTU2NzM4MjgsImxlZnQiOjUxLjE5MDQwNjc5OTMxNjQwNiwid2lkdGgiOjQxLjA2MDEyMzQ0MzYwMzUxNiwiaGVpZ2h0IjoxLjY5OTYzMTMzMzM1MTEzNTN9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo5NC45ODc5Mzc5MjcyNDYxLCJsZWZ0Ijo4LjA5NTIzNTgyNDU4NDk2MSwid2lkdGgiOjg0LjI4NTcyMDgyNTE5NTMxLCJoZWlnaHQiOjEuNjYxNzUwNjc0MjQ3NzQxN319LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjMuNzI4NDYxMjY1NTYzOTY1LCJsZWZ0Ijo4LjEwNDc2MjA3NzMzMTU0Mywid2lkdGgiOjE2LjI1ODc2OTk4OTAxMzY3MiwiaGVpZ2h0IjoxLjI2NjgxMDUzNjM4NDU4MjV9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo0LjcxOTI3MjEzNjY4ODIzMiwibGVmdCI6NzAuNDM0NzIyOTAwMzkwNjIsIndpZHRoIjoxNC4wMzA2OTAxOTMxNzYyNywiaGVpZ2h0IjoxLjMwMDc2OTMyOTA3MTA0NX19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjEwLjM3NDIwNDYzNTYyMDExNywibGVmdCI6OC4wOTUyMzU4MjQ1ODQ5NjEsIndpZHRoIjo0MS4wNzg3NTgyMzk3NDYwOTQsImhlaWdodCI6Ni45OTEyOTg2NzU1MzcxMDl9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjoxNS41OTYyMjc2NDU4NzQwMjMsImxlZnQiOjE0LjE2NzcxNDExODk1NzUyLCJ3aWR0aCI6Ny42NDA0NTYxOTk2NDU5OTYsImhlaWdodCI6MS43NjkyNzUxODg0NDYwNDV9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjoxNS41OTYyMjc2NDU4NzQwMjMsImxlZnQiOjIxLjc3NzMwOTQxNzcyNDYxLCJ3aWR0aCI6MjcuMzc3NTMyOTU4OTg0Mzc1LCJoZWlnaHQiOjEuNzY5Mjc1MTg4NDQ2MDQ1fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MTcuNDI5NzU4MDcxODk5NDE0LCJsZWZ0Ijo4LjA5NTIzNTgyNDU4NDk2MSwid2lkdGgiOjQxLjA2MDMwNjU0OTA3MjI2NiwiaGVpZ2h0IjoxLjY5OTYzMzEyMTQ5MDQ3ODV9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjoxOS4xOTM2NDU0NzcyOTQ5MjIsImxlZnQiOjguMDk1MjM1ODI0NTg0OTYxLCJ3aWR0aCI6NDEuMDY5ODY5OTk1MTE3MTksImhlaWdodCI6MS42OTk2MzMxMjE0OTA0Nzg1fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MjAuOTU3NTM0NzkwMDM5MDYyLCJsZWZ0Ijo4LjA5NTIzNTgyNDU4NDk2MSwid2lkdGgiOjQxLjA1ODc3Njg1NTQ2ODc1LCJoZWlnaHQiOjEuNjk5NjMzMTIxNDkwNDc4NX19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjIyLjcyMTQyNDEwMjc4MzIwMywibGVmdCI6OC4wOTUyMzU4MjQ1ODQ5NjEsIndpZHRoIjo0MC43MTQyOTA2MTg4OTY0ODQsImhlaWdodCI6MS42OTk2MzMxMjE0OTA0Nzg1fX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNDMwIn19XQ%3D%3D. Challenges remain in standardizing dosages across species, yet emerging evidence on finfish suggests analogous benefits, including faster smoltification in salmonids via hypoxia alleviation.

In summary, nanobubbles drive aquaculture growth promotion through DO supersaturation, metabolic acceleration, microbial modulation, and efficient nutrient utilization, surpassing conventional strategies in efficacy and cost-effectiveness. Their adoption heralds sustainable intensification, reducing FCR and environmental loads while enhancing resilience (Tekile et al., 2016)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFuZGluZXQgS2ViZWRlIiwiZmFtaWx5IjoiVGVraWxlIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikls4oCQSG8iLCJmYW1pbHkiOiJLaW0ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSmFpeWVvcCIsImZhbWlseSI6IkxlZSJ9fV0sImlkcyI6eyJpc3NuIjpbIjEyMjYtMTAyNSIsIjIwMDUtOTY4WCJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuNDQ5MS9lZXIuMjAxNi4wMjgiLCJtYWciOiIyNTIzNjY5NjU3Iiwib3BlbmFsZXgiOiJXMjUyMzY2OTY1NyJ9LCJ0aXRsZSI6IkV4dGVudCBhbmQgcGVyc2lzdGVuY2Ugb2YgZGlzc29sdmVkIG94eWdlbiBlbmhhbmNlbWVudCB1c2luZyBuYW5vYnViYmxlcyIsImFic3RyYWN0IjoiSW4gdGhpcyBzdHVkeSwgY2hhbmdlIGluIHdhdGVyLWRpc3NvbHZlZCBveHlnZW4gKERPKSB3YXMgYW5hbHl6ZWQgdW5kZXIgdmFyaW91cyBzeW50aGV0aWMgd2F0ZXIgcXVhbGl0aWVzIGFuZCBuYW5vYnViYmxlcyAoTkJzKSBhcHBsaWNhdGlvbiBjb25kaXRpb25zLCBzdWNoIGFzIGdhcyB0eXBlLCBpbml0aWFsIERPIGFzIHdlbGwgYXMgd2F0ZXIgZGlzc29sdmVkLCBzdXNwZW5kZWQgYW5kIG9yZ2FuaWMgbWF0dGVycyBjb250ZW50cy4gV2hlbiBveHlnZW4sIHJhdGhlciB0aGFuIGFpciwgd2FzIGludHJvZHVjZWQgaW50byBuaXRyb2dlbi1kZXNvcmJlZCB1bHRyYS1wdXJlIHdhdGVyLCB0aGUgc3RhZ25hdGlvbiB0aW1lIHdhcyBzaWduaWZpY2FudGx5IGluY3JlYXNlZC4gSXQgdG9vayB0ZW4gZGF5cyBmb3IgRE8gY29uY2VudHJhdGlvbiB0byBkcm9wIGJhY2sgdG8gc2F0dXJhdGlvbi4gVGhlIGhpZ2hlciB0aGUgaW5pdGlhbCBETyBjb25jZW50cmF0aW9uLCB0aGUgbG9uZ2VyIHBhcnRpY2xlcyB3ZXJlIG9ic2VydmVkIGFib3ZlIHNhdHVyYXRpb24gZHVlIHRvIHBhcnRpY2xlIHN0YWJpbGl0eSBpbXByb3ZlbWVudC4gVGhlIG94eWdlbiBtYXNzIHRyYW5zZmVyIHJhdGUgb2YgMC4wNDgyIG1nL0wvbWluIHdhcyBmb3VuZCB0byByZWFjaCBhIG1heGltdW0gYXQgYW4gZWxlY3Ryb2x5dGljIGNvbmNlbnRyYXRpb24gb2YgMC43NSBnL0wsIGJleW9uZCB3aGljaCB0aGUgdHJhbnNmZXIgcmF0ZSBkZWNyZWFzZWQgZHVlIHRvIGFkc29ycHRpb24gb2YgbmVnYXRpdmUgaW9ucyBvZiB0aGUgZWxlY3Ryb2x5dGUgYXQgdGhlIGludGVyZmFjZS4gSGlnaCBsZXZlbHMgb2YgdHVyYmlkaXR5IGNhdXNlZCBieSBzdXNwZW5kZWQgc29saWRzIGhhdmUgYmVjb21lIGEgYmFycmllciB0byBkaXNzb2x1dGlvbiBvZiBOQnMgb3h5Z2VuIGludG8gdGhlIHdhdGVyIHNvbHV0aW9uLCBhbmQgdGh1cyBhZmZlY3RlZCB0aGUgdHJhbnNmZXIgcGVyZm9ybWFuY2UuIE9uIHRoZSBvdGhlciBoYW5kLCBieSBhcHBseWluZyBOQnMgZm9yIGp1c3QgYW4gaG91ciwgdXAgdG8gNy4yJSBkZWdyYWRhdGlvbiBvZiBnbHVjb3NlIGFzIHJlcHJlc2VudGF0aXZlIG9yZ2FuaWMgbWF0dGVyIHdhcyBhY2hpZXZlZC4gVGh1cywgTkJzIHRlY2hub2xvZ3kgd291bGQgbWFpbnRhaW4gYSBoaWdoIERPIGV4dGVudCBmb3IgYW4gZXh0ZW5kZWQgZHVyYXRpb24sIGFuZCB0aHVzIGNhbiBpbXByb3ZlIHdhdGVyIHF1YWxpdHkgcHJvdmlkZWQgdGhhdCB3YXRlciBjaGVtaXN0cnkgaXMgY2xvc2VseSBtb25pdG9yZWQgZHVyaW5nIGl0cyBhcHBsaWNhdGlvbi4gS2V5d29yZHM6IEFwcGxpY2F0aW9uIGNvbmRpdGlvbnMsIERpc3NvbHZlZCBveHlnZW4sIE5hbm9idWJibGVzLCBXYXRlciBxdWFsaXR5IiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjQ0OTEvZWVyLjIwMTYuMDI4IiwicmVzb3VyY2VVcmwiOiJodHRwOi8vZWVlci5vcmcvdXBsb2FkL2Vlci0yMS00LTQyNy5wZGYiLCJpc3N1ZWREYXRlIjoiMjAxNi0wOS0xMiIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkVudmlyb25tZW50YWwgRW5naW5lZXJpbmcgUmVzZWFyY2giLCJpc3N1ZSI6NCwidm9sdW1lIjoyMSwiaW1wYWN0RmFjdG9yIjoiNC4wNSJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjQyNywibGFzdFBhZ2UiOjQzNX19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJLb3JlYW4gU29jaWV0eSBvZiBFbnZpcm9ubWVudGFsIEVuZ2luZWVyaW5nIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjoyNCwiY2l0ZWRCeUNvdW50IjozM319LCJxdW90ZSI6IlRoaXMgdmVyaWZpZXMgdGhlIHBvc3NpYmlsaXR5IG9mIHRoZSBleGlzdGVuY2UgIG9mIE5Ccywgc2luY2UgYXQgYSBoaWdoZXIgRE8gY29uY2VudHJhdGlvbiwgdGhlIG94eWdlbiBkaWZmdXNpb24gIHdvdWxkIHRha2UgbW9yZSB0aW1lLiBUaGUgaGlnaCBnYXMgY29uY2VudHJhdGlvbiBpbiB0aGUgbGlxdWlkICBleHBsYWlucyB0aGUgZXh0ZW5zaW9uIG9mIE5CIHN0YWJpbGl0eS4gIFRoZSBtYWduaXR1ZGVzIG9mIHpldGEgcG90ZW50aWFsIG9mIG94eWdlbiBuYW5vLWJ1YmJsZWQgd2F0ZXIgIHdlcmUgcG9zc2libHkgaGlnaGVyIHRoYW4gdGhvc2Ugb2YgYWlyIG5hbm8tYnViYmxlZCB3YXRlci4gQXQgIGEgaGlnaCBtYWduaXR1ZGUgemV0YSBwb3RlbnRpYWwgdmFsdWUsIHRoZSBlbGVjdHJpY2FsbHkgY2hhcmdlZCAgcGFydGljbGVzIHRlbmQgdG8gcmVwZWwgZWFjaCBvdGhlciwgYXZvaWRpbmcgYWdncmVnYXRpb24uIEZvciBidWJibGUgIGRpc3BlcnNpb24sIHRoZSBoaWdoIHpldGEgcG90ZW50aWFsIGNvdWxkIGNyZWF0ZSByZXB1bHNpb24gZm9yY2VzICB0aGF0IHdvdWxkIGF2b2lkIHRoZSBjb2FsZXNjZW5jZSBvZiBidWJibGVzLCB0aHVzIGNvbnRyaWJ1dGluZyAgdG8gdGhlIHN0YWJpbGl6YXRpb24gb2YgdGhlIGJ1YmJsZXMuIFRoZXJlZm9yZSwgdGhlIG94eWdlbiBOQnMgIGNvdWxkIGJlIHN0YWJpbGl6ZWQgaW4gd2F0ZXIgYnkgdGhlIGVsZWN0cmljYWxseSBjaGFyZ2VkIHN1cmZhY2VzLCAgd2hpbGUgdGhlIG5hbm8tYnViYmxlZCBhaXIgd291bGQgYmUgYmVsb3cgdGhlIGxpbWl0IG9mIHRoZSBzdGFiaWxpemEtdGlvbiBieSByZXB1bHNpb24gZm9yY2VzLiBGdXJ0aGVybW9yZSwgdGhlIGJpZyBkaWZmZXJlbmNlIGJldHdlZW4gdGhlIHBlcmlvZHMgaW4gd2hpY2ggIGJ1YmJsZXMgY291bGQgYmUgb2JzZXJ2ZWQgYXMgc3RhYmxlIGluIHRoZSBveHlnZW4gbmFuby1idWJibGVkICB3YXRlciAoMTAtMTUgZCkgYW5kIGluIHRoYXQgb2YgYWlyIG5hbm8tYnViYmxlZCAoMy01IGQpIGlzIGxpa2VseSBGaWcuIDYuIEdyYXBocyBzaG93aW5nIHN0YWduYXRpb24gdGltZSBmb3IgZGlmZmVyZW50IGluaXRpYWwgZGlzc29sdmVkIG94eWdlbiAoRE8pLiByZWxhdGVkIHRvIHRoZSBsb3dlciBkaXNzb2x1dGlvbiBvZiB0aGUgYWlyIGluIHdhdGVyIGluIHJlbGF0aW9uIHRvIHRoZSBkaXNzb2x1dGlvbiBvZiBveHlnZW4uICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6NjYuMTc4ODAyNDkwMjM0MzgsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6NDAuNzE0ODg5NTI2MzY3MTksImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo2Ny41NzY3NzQ1OTcxNjc5NywibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42ODgyNzA1Njg4NDc2NTYsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo2OC45NjcyMzE3NTA0ODgyOCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NTcxODA3ODYxMzI4MSwiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjcwLjM1NzY4ODkwMzgwODYsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6MjUuMzQxMjU1MTg3OTg4MjgsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3MS43NTU2NTMzODEzNDc2NiwibGVmdCI6OS41NDk2NDE2MDkxOTE4OTUsIndpZHRoIjozOS4wMjQ3ODQwODgxMzQ3NjYsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3My4xNDYxMDI5MDUyNzM0NCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NTQwMjk4NDYxOTE0MDYsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3NC41MzY1NjAwNTg1OTM3NSwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NTM0NjUyNzA5OTYwOTQsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3NS45MzQ1MzIxNjU1MjczNCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NzMyMzY4NDY5MjM4MywiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjc3LjMyNDk4OTMxODg0NzY2LCJsZWZ0Ijo3Ljg2NTU0NTI3MjgyNzE0OCwid2lkdGgiOjQwLjY0OTM1MzAyNzM0Mzc1LCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6NzguNzE1NDQ2NDcyMTY3OTcsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6NDAuNjU0NjI0OTM4OTY0ODQ0LCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODAuMTEzNDEwOTQ5NzA3MDMsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6NDAuNjUxMDUwNTY3NjI2OTUsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo4MS41MDM4NjA0NzM2MzI4MSwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NTQzNDI2NTEzNjcxOSwiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjgyLjg5NDMxNzYyNjk1MzEyLCJsZWZ0Ijo3Ljg2NTU0NTI3MjgyNzE0OCwid2lkdGgiOjQwLjMyMTgyMzEyMDExNzE5LCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODQuMjkyMjg5NzMzODg2NzIsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6MTUuODcwMjk1NTI0NTk3MTY4LCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODUuNjgyNzQ2ODg3MjA3MDMsImxlZnQiOjkuNTQ5NjQxNjA5MTkxODk1LCJ3aWR0aCI6MzguOTg5NzA0MTMyMDgwMDgsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo4Ny4wNzMyMDQwNDA1MjczNCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC43MTg0OTQ0MTUyODMyLCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODguNDcxMTY4NTE4MDY2NCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42ODc0MTk4OTEzNTc0MiwiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjM0LjUxMzI3MTMzMTc4NzExLCJsZWZ0Ijo1MS45MzI3Nzc0MDQ3ODUxNTYsIndpZHRoIjo0MC42NDcwNTI3NjQ4OTI1OCwiaGVpZ2h0IjoyLjcxODY2NjU1MzQ5NzMxNDV9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjozOC43MjUxNTg2OTE0MDYyNSwibGVmdCI6NTEuOTMyNzc3NDA0Nzg1MTU2LCJ3aWR0aCI6NDAuNjQzMTM1MDcwODAwNzgsImhlaWdodCI6MS4yNTMyODg1MDc0NjE1NDc5fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6NDAuMTE1NjEyMDMwMDI5MywibGVmdCI6NTEuOTMyNzc3NDA0Nzg1MTU2LCJ3aWR0aCI6NDAuNjY0NTg1MTEzNTI1MzksImhlaWdodCI6MS4yNTMyODg1MDc0NjE1NDc5fX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNDMxIn19XQ%3D%3D.

Plasma Water Technology

3.1. Principles and Generation Methods

Plasma-activated water represents an innovative aqueous solution produced through the interaction of liquid water with non-thermal plasma, a partially ionized gas state characterized by free electrons, ions, excited atoms, molecules, and a plethora of reactive species. Unlike thermal plasmas that reach equilibrium temperatures exceeding thousands of degrees Kelvin, non-thermal plasmas maintain electron temperatures in the range of 1–10 eV while keeping the gas temperature near ambient levels (around 300–400 K). This disparity arises from the low collision rates between heavy particles and the high-energy electrons, enabling selective energy transfer primarily to electrons. In PAW generation, plasma is typically discharged directly over or within water, leading to the dissolution of short-lived plasma-generated species into the liquid phase. These species, including reactive oxygen and nitrogen species, hydrogen peroxide (H₂O₂), ozone (O₃), and nitrite (NO₂⁻)/nitrate (NO₃⁻) ions, confer PAW its potent physicochemical properties, such as low pH (often 2–4), high oxidation-reduction potential (ORP > 400 mV), and elevated electrical conductivity.

Various methods are employed to generate non-thermal plasma for PAW production, each tailored to specific scalability, energy efficiency, and species yield requirements. Figure 2 presents a schematic of plasma-activated water generation using a dielectric barrier discharge configuration. Dielectric barrier discharge is among the most common, involving two electrodes separated by a dielectric material (e.g., quartz or alumina) and a gas-filled gap exposed to water. High-voltage alternating current (5–20 kV, 1–50 kHz) creates microdischarges that propagate through the gap, ionizing the gas and producing radicals via electron-impact dissociation of O₂ and N₂ molecules. Corona discharge, simpler and electrode-based, utilizes a sharp electrode (e.g., wire or needle) in ambient air or humidified gas adjacent to water, where intense electric fields (>30 kV/cm) accelerate electrons to initiate avalanches. Gliding arc discharge offers higher power throughput by sustaining a plasma arc that glides along diverging electrodes, combining thermal and non-thermal characteristics for voluminous reactive species production. Emerging techniques include plasma jets (kINPen-like devices using radiofrequency power), underwater plasma via pulsed discharges, and cavitation-assisted plasmas, which enhance species transfer through bubble implosions. Selection depends on application: DBD suits continuous flow systems, while gliding arcs excel in batch processing for aquaculture volumes.

Figure 2: Schematic of plasma-activated water generation using dielectric barrier discharge

The core mechanism of PAW formation hinges on plasma-water interfacial dynamics and volumetric dissolution processes. Plasma electrons and UV photons bombard the water surface, inducing photoionization and ejection of solvated electrons (H₂O → H₂O⁺ + e⁻_aq), which react rapidly with dissolved O₂ to form superoxide (O₂⁻•) and subsequently H₂O₂ via disproportionation. Gas-phase radicals like •OH, •O, and NO• diffuse into the liquid, where they undergo hydration (e.g., •OH + H₂O → H₂O₂ + •H) or react with ambient humidity. Nitrogen fixation yields NO₂⁻ and NO₃⁻ through multistep pathways: N₂ + O• → NO + N, followed by oxidation cascades. Bubble-mediated transport, prevalent in submerged or indirect exposures, amplifies transfer efficiency, as collapsing microbubbles generate localized hotspots mirroring sonochemical effects. The resulting PAW cocktail-typically 1–100 μM H₂O₂, 0.1–10 mM NO₂⁻/NO₃⁻, and transient •OH (detectable via EPR)-persists for hours to days, with longevity tunable by post-plasma storage under controlled atmospheres. Key physicochemical properties of plasma-activated water relevant to aquaculture performance are summarized in Table 3.

Table 3: Properties of plasma-activated water

| Property | Description |
| --- | --- |
| Reactive species | H2O2, NO2-, RONS |
| pH | Acidic (pH 2-6) |
| Oxidation-reduction potential | High ORP (>1000 mV) |
| Antimicrobial activity | Effective against bacteria, viruses, fungi |

In aquaculture contexts, PAW generation aligns seamlessly with intensive systems, mirroring nanobubble scalability discussed previously. Compact DBD reactors integrated into recirculating aquaculture systems treat water volumes up to 10 m³/h at energy costs of 1–5 kWh/m³, producing PAW with tailored RONS profiles for disinfection without chemical residues. For example, corona-based setups achieve greater than 5-log inactivation of Vibrio spp. in shrimp ponds, complementing the biophysical actions of oxygen nanobubbles. Parameters such as input gas (air, O₂, Ar), power density (1–100 W/cm²), and treatment time (1–30 min) optimize species balance: O₂-enriched feeds increase H₂O₂/O₃ for oxidation, while air plasmas focus on nitrogenous compounds for pH modulation and nutrient stimulation. Safety considerations include ozone residuals (<0.1 ppm) and electrode erosion mitigation via coatings.

Overall, PAW's principles leverage plasma's non-equilibrium nature to engineer bioactive water, with generation methods offering versatility from lab-scale to industrial deployment. This technology bridges physical plasma chemistry with aqueous biology, paving the way for sustainable aquaculture enhancements in disinfection, immunity, and growth, as explored in subsequent sections.

3.2. Properties and Mechanisms of Action

Plasma-activated water exhibits a unique set of physicochemical properties that underpin its biological efficacy, particularly in aquaculture applications. These properties result directly from the plasma-water interaction described earlier, generating a complex mix of reactive oxygen and nitrogen species, hydrogen peroxide (H₂O₂), ozone (O₃), nitrite (NO₂⁻), nitrate (NO₃⁻) ions, as well as changes in pH, oxidation-reduction potential (ORP), and electrical conductivity. Typically, PAW has a low pH, ranging from 2.0 to 5.5, due to the formation of nitric and nitrous acids from nitrogen fixation processes. ORP values often exceed +400 mV, indicating a highly oxidative environment that promotes disinfection. Electrical conductivity can rise to 1–10 mS/cm due to the accumulation of ionic species. Concentrations vary by generation method: H₂O₂ at 1–100 μM, NO₂⁻/NO₃⁻ at 0.1–10 mM, and transient radicals such as hydroxyl (•OH) at trace levels. These attributes persist for hours to days, with stability enhanced by storing in opaque, sealed containers to minimize UV degradation and re-equilibration. In aquaculture, these properties allow PAW to outperform traditional disinfectants by providing residue-free treatment, as reactive species decompose into benign byproducts such as water and nitrates, which can even serve as bioavailable nitrogen.

The reactive species in PAW drive its mechanisms of action through oxidative stress induction and targeted biochemical interactions. H₂O₂, a stable long-lived species, diffuses across microbial membranes, reacting with iron-sulfur clusters in enzymes via Fenton chemistry to generate highly reactive •OH intracellularly. This leads to lipid peroxidation, protein carbonylation, and DNA strand breaks, culminating in microbial inactivation. RONS such as peroxynitrite (ONOO⁻), formed from NO• and O₂•⁻, exhibit synergistic potency, attacking thiol groups in proteins and disrupting quorum sensing in biofilms. Nitrite ions contribute via acid-catalyzed formation of nitrous acid (HNO₂), a potent nitrosating agent that inhibits bacterial respiration by targeting cytochrome oxidases. The acidic pH amplifies these effects by protonating cellular membranes, increasing permeability and facilitating species ingress. Unlike broad-spectrum antibiotics, PAW's multi-target approach minimizes resistance development, as evidenced by >5-log reductions in pathogens such as Vibrio parahaemolyticus and Aeromonas hydrophila at sub-lethal doses that spare host cells

Beyond antimicrobial activity, PAW elicits adaptive cellular responses in aquatic organisms, promoting health benefits. In fish and shellfish, low-dose PAW exposure triggers hormesis—a biphasic dose-response in which mild stress upregulates antioxidant defenses, including superoxide dismutase, catalase, and glutathione peroxidase. This enhances resilience against oxidative stress caused by high stocking densities or poor water quality. Nitrogen species modulate signaling pathways: NO₂⁻ acts as a reservoir for nitric oxide (NO), activating guanylate cyclase to promote vasodilation and immune cell recruitment. Studies indicate that PAW immersion boosts lysozyme activity and IgM levels in tilapia, correlating with a 20–30% increase in survival following an Edwardsiella tarda challenge. Wound healing is accelerated via H₂O₂-mediated macrophage polarization and fibroblast proliferation, reducing scarring in fin injuries.

PAW also influences growth and metabolism through nutrient-like effects. Nitrate and nitrite assimilation by microalgae stimulates chlorophyll synthesis and biomass yield, indirectly supporting zooplankton and larval feeds in integrated systems. Direct exposure enhances feed utilization by altering gut microbiota, favoring probiotics like Lactobacillus over pathogens, thus lowering feed conversion ratios by 10–15%. Metabolic acceleration mirrors nanobubble effects (Tekile et al., 2016)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFuZGluZXQgS2ViZWRlIiwiZmFtaWx5IjoiVGVraWxlIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikls4oCQSG8iLCJmYW1pbHkiOiJLaW0ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSmFpeWVvcCIsImZhbWlseSI6IkxlZSJ9fV0sImlkcyI6eyJpc3NuIjpbIjEyMjYtMTAyNSIsIjIwMDUtOTY4WCJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuNDQ5MS9lZXIuMjAxNi4wMjgiLCJtYWciOiIyNTIzNjY5NjU3Iiwib3BlbmFsZXgiOiJXMjUyMzY2OTY1NyJ9LCJ0aXRsZSI6IkV4dGVudCBhbmQgcGVyc2lzdGVuY2Ugb2YgZGlzc29sdmVkIG94eWdlbiBlbmhhbmNlbWVudCB1c2luZyBuYW5vYnViYmxlcyIsImFic3RyYWN0IjoiSW4gdGhpcyBzdHVkeSwgY2hhbmdlIGluIHdhdGVyLWRpc3NvbHZlZCBveHlnZW4gKERPKSB3YXMgYW5hbHl6ZWQgdW5kZXIgdmFyaW91cyBzeW50aGV0aWMgd2F0ZXIgcXVhbGl0aWVzIGFuZCBuYW5vYnViYmxlcyAoTkJzKSBhcHBsaWNhdGlvbiBjb25kaXRpb25zLCBzdWNoIGFzIGdhcyB0eXBlLCBpbml0aWFsIERPIGFzIHdlbGwgYXMgd2F0ZXIgZGlzc29sdmVkLCBzdXNwZW5kZWQgYW5kIG9yZ2FuaWMgbWF0dGVycyBjb250ZW50cy4gV2hlbiBveHlnZW4sIHJhdGhlciB0aGFuIGFpciwgd2FzIGludHJvZHVjZWQgaW50byBuaXRyb2dlbi1kZXNvcmJlZCB1bHRyYS1wdXJlIHdhdGVyLCB0aGUgc3RhZ25hdGlvbiB0aW1lIHdhcyBzaWduaWZpY2FudGx5IGluY3JlYXNlZC4gSXQgdG9vayB0ZW4gZGF5cyBmb3IgRE8gY29uY2VudHJhdGlvbiB0byBkcm9wIGJhY2sgdG8gc2F0dXJhdGlvbi4gVGhlIGhpZ2hlciB0aGUgaW5pdGlhbCBETyBjb25jZW50cmF0aW9uLCB0aGUgbG9uZ2VyIHBhcnRpY2xlcyB3ZXJlIG9ic2VydmVkIGFib3ZlIHNhdHVyYXRpb24gZHVlIHRvIHBhcnRpY2xlIHN0YWJpbGl0eSBpbXByb3ZlbWVudC4gVGhlIG94eWdlbiBtYXNzIHRyYW5zZmVyIHJhdGUgb2YgMC4wNDgyIG1nL0wvbWluIHdhcyBmb3VuZCB0byByZWFjaCBhIG1heGltdW0gYXQgYW4gZWxlY3Ryb2x5dGljIGNvbmNlbnRyYXRpb24gb2YgMC43NSBnL0wsIGJleW9uZCB3aGljaCB0aGUgdHJhbnNmZXIgcmF0ZSBkZWNyZWFzZWQgZHVlIHRvIGFkc29ycHRpb24gb2YgbmVnYXRpdmUgaW9ucyBvZiB0aGUgZWxlY3Ryb2x5dGUgYXQgdGhlIGludGVyZmFjZS4gSGlnaCBsZXZlbHMgb2YgdHVyYmlkaXR5IGNhdXNlZCBieSBzdXNwZW5kZWQgc29saWRzIGhhdmUgYmVjb21lIGEgYmFycmllciB0byBkaXNzb2x1dGlvbiBvZiBOQnMgb3h5Z2VuIGludG8gdGhlIHdhdGVyIHNvbHV0aW9uLCBhbmQgdGh1cyBhZmZlY3RlZCB0aGUgdHJhbnNmZXIgcGVyZm9ybWFuY2UuIE9uIHRoZSBvdGhlciBoYW5kLCBieSBhcHBseWluZyBOQnMgZm9yIGp1c3QgYW4gaG91ciwgdXAgdG8gNy4yJSBkZWdyYWRhdGlvbiBvZiBnbHVjb3NlIGFzIHJlcHJlc2VudGF0aXZlIG9yZ2FuaWMgbWF0dGVyIHdhcyBhY2hpZXZlZC4gVGh1cywgTkJzIHRlY2hub2xvZ3kgd291bGQgbWFpbnRhaW4gYSBoaWdoIERPIGV4dGVudCBmb3IgYW4gZXh0ZW5kZWQgZHVyYXRpb24sIGFuZCB0aHVzIGNhbiBpbXByb3ZlIHdhdGVyIHF1YWxpdHkgcHJvdmlkZWQgdGhhdCB3YXRlciBjaGVtaXN0cnkgaXMgY2xvc2VseSBtb25pdG9yZWQgZHVyaW5nIGl0cyBhcHBsaWNhdGlvbi4gS2V5d29yZHM6IEFwcGxpY2F0aW9uIGNvbmRpdGlvbnMsIERpc3NvbHZlZCBveHlnZW4sIE5hbm9idWJibGVzLCBXYXRlciBxdWFsaXR5IiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjQ0OTEvZWVyLjIwMTYuMDI4IiwicmVzb3VyY2VVcmwiOiJodHRwOi8vZWVlci5vcmcvdXBsb2FkL2Vlci0yMS00LTQyNy5wZGYiLCJpc3N1ZWREYXRlIjoiMjAxNi0wOS0xMiIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkVudmlyb25tZW50YWwgRW5naW5lZXJpbmcgUmVzZWFyY2giLCJpc3N1ZSI6NCwidm9sdW1lIjoyMSwiaW1wYWN0RmFjdG9yIjoiNC4wNSJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjQyNywibGFzdFBhZ2UiOjQzNX19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJLb3JlYW4gU29jaWV0eSBvZiBFbnZpcm9ubWVudGFsIEVuZ2luZWVyaW5nIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjoyNCwiY2l0ZWRCeUNvdW50IjozM319LCJxdW90ZSI6IlRoaXMgdmVyaWZpZXMgdGhlIHBvc3NpYmlsaXR5IG9mIHRoZSBleGlzdGVuY2UgIG9mIE5Ccywgc2luY2UgYXQgYSBoaWdoZXIgRE8gY29uY2VudHJhdGlvbiwgdGhlIG94eWdlbiBkaWZmdXNpb24gIHdvdWxkIHRha2UgbW9yZSB0aW1lLiBUaGUgaGlnaCBnYXMgY29uY2VudHJhdGlvbiBpbiB0aGUgbGlxdWlkICBleHBsYWlucyB0aGUgZXh0ZW5zaW9uIG9mIE5CIHN0YWJpbGl0eS4gIFRoZSBtYWduaXR1ZGVzIG9mIHpldGEgcG90ZW50aWFsIG9mIG94eWdlbiBuYW5vLWJ1YmJsZWQgd2F0ZXIgIHdlcmUgcG9zc2libHkgaGlnaGVyIHRoYW4gdGhvc2Ugb2YgYWlyIG5hbm8tYnViYmxlZCB3YXRlci4gQXQgIGEgaGlnaCBtYWduaXR1ZGUgemV0YSBwb3RlbnRpYWwgdmFsdWUsIHRoZSBlbGVjdHJpY2FsbHkgY2hhcmdlZCAgcGFydGljbGVzIHRlbmQgdG8gcmVwZWwgZWFjaCBvdGhlciwgYXZvaWRpbmcgYWdncmVnYXRpb24uIEZvciBidWJibGUgIGRpc3BlcnNpb24sIHRoZSBoaWdoIHpldGEgcG90ZW50aWFsIGNvdWxkIGNyZWF0ZSByZXB1bHNpb24gZm9yY2VzICB0aGF0IHdvdWxkIGF2b2lkIHRoZSBjb2FsZXNjZW5jZSBvZiBidWJibGVzLCB0aHVzIGNvbnRyaWJ1dGluZyAgdG8gdGhlIHN0YWJpbGl6YXRpb24gb2YgdGhlIGJ1YmJsZXMuIFRoZXJlZm9yZSwgdGhlIG94eWdlbiBOQnMgIGNvdWxkIGJlIHN0YWJpbGl6ZWQgaW4gd2F0ZXIgYnkgdGhlIGVsZWN0cmljYWxseSBjaGFyZ2VkIHN1cmZhY2VzLCAgd2hpbGUgdGhlIG5hbm8tYnViYmxlZCBhaXIgd291bGQgYmUgYmVsb3cgdGhlIGxpbWl0IG9mIHRoZSBzdGFiaWxpemEtdGlvbiBieSByZXB1bHNpb24gZm9yY2VzLiBGdXJ0aGVybW9yZSwgdGhlIGJpZyBkaWZmZXJlbmNlIGJldHdlZW4gdGhlIHBlcmlvZHMgaW4gd2hpY2ggIGJ1YmJsZXMgY291bGQgYmUgb2JzZXJ2ZWQgYXMgc3RhYmxlIGluIHRoZSBveHlnZW4gbmFuby1idWJibGVkICB3YXRlciAoMTAtMTUgZCkgYW5kIGluIHRoYXQgb2YgYWlyIG5hbm8tYnViYmxlZCAoMy01IGQpIGlzIGxpa2VseSBGaWcuIDYuIEdyYXBocyBzaG93aW5nIHN0YWduYXRpb24gdGltZSBmb3IgZGlmZmVyZW50IGluaXRpYWwgZGlzc29sdmVkIG94eWdlbiAoRE8pLiByZWxhdGVkIHRvIHRoZSBsb3dlciBkaXNzb2x1dGlvbiBvZiB0aGUgYWlyIGluIHdhdGVyIGluIHJlbGF0aW9uIHRvIHRoZSBkaXNzb2x1dGlvbiBvZiBveHlnZW4uICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6NjYuMTc4ODAyNDkwMjM0MzgsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6NDAuNzE0ODg5NTI2MzY3MTksImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo2Ny41NzY3NzQ1OTcxNjc5NywibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42ODgyNzA1Njg4NDc2NTYsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo2OC45NjcyMzE3NTA0ODgyOCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NTcxODA3ODYxMzI4MSwiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjcwLjM1NzY4ODkwMzgwODYsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6MjUuMzQxMjU1MTg3OTg4MjgsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3MS43NTU2NTMzODEzNDc2NiwibGVmdCI6OS41NDk2NDE2MDkxOTE4OTUsIndpZHRoIjozOS4wMjQ3ODQwODgxMzQ3NjYsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3My4xNDYxMDI5MDUyNzM0NCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NTQwMjk4NDYxOTE0MDYsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3NC41MzY1NjAwNTg1OTM3NSwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NTM0NjUyNzA5OTYwOTQsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3NS45MzQ1MzIxNjU1MjczNCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NzMyMzY4NDY5MjM4MywiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjc3LjMyNDk4OTMxODg0NzY2LCJsZWZ0Ijo3Ljg2NTU0NTI3MjgyNzE0OCwid2lkdGgiOjQwLjY0OTM1MzAyNzM0Mzc1LCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6NzguNzE1NDQ2NDcyMTY3OTcsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6NDAuNjU0NjI0OTM4OTY0ODQ0LCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODAuMTEzNDEwOTQ5NzA3MDMsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6NDAuNjUxMDUwNTY3NjI2OTUsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo4MS41MDM4NjA0NzM2MzI4MSwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NTQzNDI2NTEzNjcxOSwiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjgyLjg5NDMxNzYyNjk1MzEyLCJsZWZ0Ijo3Ljg2NTU0NTI3MjgyNzE0OCwid2lkdGgiOjQwLjMyMTgyMzEyMDExNzE5LCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODQuMjkyMjg5NzMzODg2NzIsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6MTUuODcwMjk1NTI0NTk3MTY4LCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODUuNjgyNzQ2ODg3MjA3MDMsImxlZnQiOjkuNTQ5NjQxNjA5MTkxODk1LCJ3aWR0aCI6MzguOTg5NzA0MTMyMDgwMDgsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo4Ny4wNzMyMDQwNDA1MjczNCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC43MTg0OTQ0MTUyODMyLCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODguNDcxMTY4NTE4MDY2NCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42ODc0MTk4OTEzNTc0MiwiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjM0LjUxMzI3MTMzMTc4NzExLCJsZWZ0Ijo1MS45MzI3Nzc0MDQ3ODUxNTYsIndpZHRoIjo0MC42NDcwNTI3NjQ4OTI1OCwiaGVpZ2h0IjoyLjcxODY2NjU1MzQ5NzMxNDV9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjozOC43MjUxNTg2OTE0MDYyNSwibGVmdCI6NTEuOTMyNzc3NDA0Nzg1MTU2LCJ3aWR0aCI6NDAuNjQzMTM1MDcwODAwNzgsImhlaWdodCI6MS4yNTMyODg1MDc0NjE1NDc5fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6NDAuMTE1NjEyMDMwMDI5MywibGVmdCI6NTEuOTMyNzc3NDA0Nzg1MTU2LCJ3aWR0aCI6NDAuNjY0NTg1MTEzNTI1MzksImhlaWdodCI6MS4yNTMyODg1MDc0NjE1NDc5fX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNDMxIn19XQ%3D%3D, with elevated ORP optimizing electron transport chain efficiency in mitochondria. However, dose optimization is critical: excessive RONS (>500 μM H₂O₂) can induce apoptosis, highlighting the need for real-time monitoring via ORP/pH probes.

In summary, PAW's properties—low pH, high ORP, and RONS profile—mediate multifaceted mechanisms, from disinfection to immunostimulation and growth promotion. This versatility positions PAW as a tunable bioagent, with efficacy scalable via plasma parameters, paving the way for its integration into sustainable aquaculture alongside complementary technologies like oxygen nanobubbles. These nanobubbles enhance gas transfer efficiency and maintain dissolved oxygen levels, synergizing with the oxidative potential of PAW to support aerobic microbial processes and fish respiration (Vichiansan et al., 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5vcnJhcG9uIiwiZmFtaWx5IjoiVmljaGlhbnNhbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQb3Jua2FyYW4iLCJmYW1pbHkiOiJQaGFuLXVtb25nIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRoaXBwYXdhbiIsImZhbWlseSI6Ik1la3dpbGFpIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkphdHVyb25nIiwiZmFtaWx5IjoiS3VtbGEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiS29tZ3JpdCIsImZhbWlseSI6Ikxla3Nha3VsIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNhaXNhbW9ybiIsImZhbWlseSI6Ikx1bXlvbmcifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiRGhlZXJhd2FuIiwiZmFtaWx5IjoiQm9vbnlhd2FuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlBpcGF0aCIsImZhbWlseSI6IlBvcmFtYXBpaml0d2F0In19XSwiaWRzIjp7Imlzc24iOltdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMjEyMDMvcnMuMy5ycy0zMjMwNTYxL3YxIiwib3BlbmFsZXgiOiJXNDM4NTk1NDk1MyIsIm9wZW5BY2Nlc3NJZCI6IjE2MTA0MDEzOTgzMzYwMzE3NzA2In0sInRpdGxlIjoiQXBwcm9wcmlhdGUgUGFyYW1ldGVycyBmb3IgZ3Jvd3RoIFJlZHVjdGlvbiBvZiBtaWNyb29yZ2FuaXNtcyBpbiBXaGl0ZSBsZWcgU2hyaW1wIChMaXRvcGVuYWV1cyB2YW5uYW1laSkgYW5kIFNwbGVuZGlkIFNxdWlkIChMb2xpZ28gZm9ybW9zYW5hKSBieSBQbGFzbWEgQWN0aXZhdGVkIFdhdGVyIiwiYWJzdHJhY3QiOiJBYnN0cmFjdCBUaGUgc3R1ZHkgZm9jdXNlcyBvbiB0aGUgYXBwbGljYXRpb24gb2YgUEFXIGluYWN0aXZhdGlvbiBmb3IgZXhwb3J0aW5nIHdoaXRlIGxlZyBzaHJpbXAgYW5kIHNwbGVuZGlkIHNxdWlkLiBUaGUgZXhwZXJpbWVudCBkZXRlcm1pbmVkIHRoZSBtYXhpbXVtIGh5ZHJvZ2VuIHBlcm94aWRlIGNvbmNlbnRyYXRpb24gaW4gUEFXIGJ5IHV0aWxpemluZyBhIGZ1bGwgZmFjdG9yaWFsIGRlc2lnbiB3aXRoIHR3by1sZXZlbCBmYWN0b3JzLiBTdWJzZXF1ZW50bHksIHRoZSBhbmFseXNpcyBvZiB2YXJpYW5jZSAoQU5PVkEpIHdhcyBjb25kdWN0ZWQgdXNpbmcgTWluaXRhYi4gVGhlIHJlc3VsdHMgaW5kaWNhdGVkIHRoYXQgYSBnYXMgZmxvdyByYXRlIG9mIGEgbWl4dHVyZSBvZiBBciAoOTglKSBhbmQgTyAyICgyJSkgYXQgMjUgTC9taW4sIHdpdGggYSBkaXNjaGFyZ2UgdGltZSBvZiAxMjAgbWludXRlcywgZ2VuZXJhdGVkIHRoZSBvcHRpbWFsIGNvbmNlbnRyYXRpb24gb2YgSCAyIE8gMiBpbiBQQVcgYXQgMTMuMzAgbWcvTCwgYXMgcGVyIHRoZSByZXNwb25zZSBvcHRpbWl6ZXIuIFRoZSB1c2Ugb2YgUEFXIHRvIGluaGliaXQgdGhlIGdyb3d0aCBvZiBtaWNyb29yZ2FuaXNtcyBpbiB3aGl0ZSBsZWcgc2hyaW1wIGFuZCBzcGxlbmRpZCBzcXVpZCBkZW1vbnN0cmF0ZWQgdGhhdCBpbmFjdGl2YXRpb24gd2l0aCBQQVcsIHNvYWtlZCBmb3IgMTAgbWludXRlcywgcmVzdWx0ZWQgaW4gYSByZWR1Y3Rpb24gb2YgdG90YWwgbWljcm9vcmdhbmlzbXMgdG8gbGVzcyB0aGFuIDYgTG9nIENGVS9nLiBBZGRpdGlvbmFsbHksIGFuIGV4YW1pbmF0aW9uIG9mIHRoZSBudXRyaXRpb24gb2YgYXF1YWN1bHR1cmUgZm9vZCByZXZlYWxlZCB0aGF0IFBBVy1pbW1lcnNlZCB3aGl0ZSBsZWcgc2hyaW1wIGFuZCBzcGxlbmRpZCBzcXVpZCBzaG93ZWQgZGlmZmVyZW50IGVmZmVjdHMgb24gcHJvdGVpbiBhbmQgbGlwaWQgY2hhbmdlcy4gVGhlIHJlc3VsdHMgc3VnZ2VzdCB0aGF0IFBBVyBjb3VsZCBiZSBhIHZpYWJsZSBvcHRpb24gZm9yIGRlY29udGFtaW5hdGluZyBhcXVhdGljIGZvb2QgcHJvZHVjdHMuIEhvd2V2ZXIsIGl0IGlzIGltcG9ydGFudCB0byBjb25zaWRlciB0aGF0IG94aWRhdGlvbiBtYXkgcG9zZSBhIGNoYWxsZW5nZSBpbiBjZXJ0YWluIHByb2R1Y3RzLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4yMTIwMy9ycy4zLnJzLTMyMzA1NjEvdjEiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vd3d3LnJlc2VhcmNoc3F1YXJlLmNvbS9hcnRpY2xlL3JzLTMyMzA1NjEvbGF0ZXN0LnBkZiIsImlzc3VlZERhdGUiOiIyMDIzLTA4LTE3IiwiYWNjZXNzZWREYXRlIjoiMjAyNS0wOCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiUmVzZWFyY2ggU3F1YXJlIChSZXNlYXJjaCBTcXVhcmUpIiwiaW1wYWN0RmFjdG9yIjoiMC40MCJ9LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJSZXNlYXJjaCBTcXVhcmUgKFVuaXRlZCBTdGF0ZXMpIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo0MCwiY2l0ZWRCeUNvdW50IjowfX0sInF1b3RlIjoiSHlkcm9nZW4gcGVyb3hpZGUgcHJvZHVjdGlvbjogTysgSDIgT%2BKGkiBPSOKLhSBPSOKLhStPSOKLheKGkkgyTzIgKDMpIE5pdHJhdGUgcHJvZHVjdGlvbjogTjIgKyBPMiDihpIgMk5PICg0KSAyTk8gKyBPMiDihpIgMk5PMiAoNSkgM05PMiArIEgyIE%2FihpIgMkhOTzMgKyBOTyAoNikgSDJPMiBpcyBhIHJlbGF0aXZlbHkgc3RhYmxlIGNvbXBvdW5kLCBhbmQgaXRzIGJlaGF2aW9yIGluIGxpcXVpZCBlbnZpcm9ubWVudHMgY2FuIGxlYWQgdG8gdGhlIGZvcm1hdGlvbiBvZiBoeWRyb3Blcm94eSByYWRpY2FscyBvciBzdXBlcm94aWRlcywgb3IgZXZlbiBoeWRyb3h5bCByYWRpY2Fscy4gUmVzZWFyY2ggY29uZHVjdGVkIGJ5IE5pcXVldCBldCBhbC4gKDIwMTgpIHVzaW5nIGEgZGllbGVjdHJpYyBwbGFzbWEgc3lzdGVtIGRlbW9uc3RyYXRlZCB0aGF0IEgyTzIgY29uY2VudHJhdGlvbnMgaW4gUEFXIGluY3JlYXNlZCB3aXRoIGxvbmdlciBwbGFzbWEgdHJlYXRtZW50IHRpbWVzIGFuZCByZW1haW5lZCBzdGFibGUgZHVyaW5nIGV4dGVuZGVkIHN0b3JhZ2UgcGVyaW9kcy4gSDJPMiBpcyBrbm93biBmb3IgaXRzIHNpZ25p77%2B9Y2FudCBhbnRpbWljcm9iaWFsIGFuZCBjeXRvdG94aWMgcHJvcGVydGllcyBpbiBwbGFzbWEtYWN0aXZhdGVkIGxpcXVpZHMgKEp1bMOhaywgSHVqYWNvdsOhIGV0IGFsLiwgMjAxOCkuIFRoZSBleHBlcmltZW50IHJldmVhbGVkIHRoYXQgdGhlIGRlY29tcG9zaXRpb24gcmF0ZSBvZiBIMk8yIGlzIHRlbXBlcmF0dXJlLWRlcGVuZGVudCwgd2l0aCBoaWdoZXIgdGVtcGVyYXR1cmVzIGxlYWRpbmcgdG8gYSBkZWNyZWFzZSBpbiBIMk8yIGNvbmNlbnRyYXRpb24uIENvbnZlcnNlbHksIGxvd2VyIHRlbXBlcmF0dXJlcyBmYWNpbGl0YXRlIHRoZSBtYWludGVuYW5jZSBvZiBIMk8yIGNvbmNlbnRyYXRpb24gb3ZlciBhIGxvbmdlciBkdXJhdGlvbi4gVGhpcyBpbmRpY2F0ZXMgdGhhdCBzdG9yaW5nIFBBVyBhdCBsb3cgdGVtcGVyYXR1cmVzIGJldHRlciBwcmVzZXJ2ZXMgdGhlIGNvbmNlbnRyYXRpb24gb2YgSDJPMiBjb21wYXJlZCB0byBzdG9yaW5nIGl0IGF0IGhpZ2ggdGVtcGVyYXR1cmVzLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjoyMC40ODM2NzUwMDMwNTE3NTgsImxlZnQiOjQuNTc1MTYyODg3NTczMjQyLCJ3aWR0aCI6MjYuNDc5MDM4MjM4NTI1MzksImhlaWdodCI6MS43NzcxNjMzODYzNDQ5MDk3fX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjI0LjQwMTkwMTI0NTExNzE4OCwibGVmdCI6NC41NzUxNjI4ODc1NzMyNDIsIndpZHRoIjozMC4yNDMxNDExNzQzMTY0MDYsImhlaWdodCI6MS44Mzk3ODYwNTI3MDM4NTc0fX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjI4LjI1NTc5MjYxNzc5Nzg1LCJsZWZ0Ijo0LjU3NTE2Mjg4NzU3MzI0Miwid2lkdGgiOjE2LjA1MzAxMjg0NzkwMDM5LCJoZWlnaHQiOjEuNzc3MTYzMzg2MzQ0OTA5N319LHsicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjozMi4wNDcwNjE5MjAxNjYwMTYsImxlZnQiOjQuNTc1MTYyODg3NTczMjQyLCJ3aWR0aCI6MjcuNjY3Mjk5MjcwNjI5ODgzLCJoZWlnaHQiOjYuMzI2Njg3MzM1OTY4MDE4fX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjQwLjM4NzkyMDM3OTYzODY3LCJsZWZ0Ijo0LjU3NTE2Mjg4NzU3MzI0Miwid2lkdGgiOjkwLjcxMzExOTUwNjgzNTk0LCJoZWlnaHQiOjIuMDYzNzAxMTUyODAxNTEzN319LHsicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjo0Mi44NTIxOTU3Mzk3NDYwOTQsImxlZnQiOjQuNTc1MTYyODg3NTczMjQyLCJ3aWR0aCI6ODUuMzk2NjIxNzA0MTAxNTYsImhlaWdodCI6MS43NzcxNjMzODYzNDQ5MDk3fX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjQ1LjEyNzAyMTc4OTU1MDc4LCJsZWZ0Ijo0LjU3NTE2Mjg4NzU3MzI0Miwid2lkdGgiOjg4Ljc4MjIxMTMwMzcxMDk0LCJoZWlnaHQiOjIuMDYzNzAxMTUyODAxNTEzN319LHsicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjo0Ny41OTEyOTcxNDk2NTgyLCJsZWZ0Ijo0LjU3NTE2Mjg4NzU3MzI0Miwid2lkdGgiOjg5LjQ2OTI5MTY4NzAxMTcyLCJoZWlnaHQiOjIuMDYzNzYyNjY0Nzk0OTIyfX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjUwLjA1NTY0MTE3NDMxNjQwNiwibGVmdCI6NC41NzUxNjI4ODc1NzMyNDIsIndpZHRoIjo4OC41NzcxNDA4MDgxMDU0NywiaGVpZ2h0Ijo0LjMzODUyNDM0MTU4MzI1Mn19LHsicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjo1NC43OTQ3Mzg3Njk1MzEyNSwibGVmdCI6NC41NzUxNjI4ODc1NzMyNDIsIndpZHRoIjo4My4zMTYyMDc4ODU3NDIxOSwiaGVpZ2h0IjoyLjA2Mzc2MjY2NDc5NDkyMn19LHsicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjo1Ny4yNTkwODI3OTQxODk0NSwibGVmdCI6NC41NzUxNjI4ODc1NzMyNDIsIndpZHRoIjo4OS41NjUxMjQ1MTE3MTg3NSwiaGVpZ2h0IjoyLjA2Mzc2MjY2NDc5NDkyMn19LHsicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjo1OS43MjM0MjY4MTg4NDc2NTYsImxlZnQiOjQuNTc1MTYyODg3NTczMjQyLCJ3aWR0aCI6ODAuMDczNDEwMDM0MTc5NjksImhlaWdodCI6Mi4wNjM3NjI2NjQ3OTQ5MjJ9fSx7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6NjIuMTg3NzY3MDI4ODA4NTk0LCJsZWZ0Ijo0LjU3NTE2Mjg4NzU3MzI0Miwid2lkdGgiOjExLjkyMzgwOTA1MTUxMzY3MiwiaGVpZ2h0IjoxLjc3NzE2MzM4NjM0NDkwOTd9fV19LCJjaHVua0lkIjoiNDllYzVmNGItOGJkNy00ZTJiLWI0N2QtN2NjYWQzNjg1NzFlIiwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiMTEifX1d.

3.3. Applications in Aquaculture

3.3.1. Water Disinfection

Plasma-activated water demonstrates potent disinfection capabilities against a wide range of aquaculture-relevant microorganisms, including Gram-positive and Gram-negative bacteria (e.g., Vibrio parahaemolyticus, Aeromonas hydrophila, Edwardsiella tarda), viruses, fungi, and protozoan parasites. This broad-spectrum efficacy stems from the synergistic action of reactive oxygen and nitrogen species, including hydrogen peroxide (H₂O₂), nitrite (NO₂⁻), nitrate (NO₃⁻), ozone (O₃), and short-lived radicals like hydroxyl (•OH). Unlike conventional chemical disinfectants such as chlorine or quaternary ammonium compounds, PAW achieves microbial inactivation without leaving harmful residues, as its reactive components decompose into water, oxygen, and nitrates—nutrients that can benefit aquatic ecosystems. Studies show consistent reductions of >5 logs in bacterial loads within minutes to hours of exposure, even at sub-lethal concentrations that preserve eukaryotic host cells. This selective toxicity stems from prokaryotic vulnerabilities, such as thinner cell walls and higher metabolic rates, making PAW ideal for recirculating aquaculture systems, where water quality directly impacts fish health and productivity.

The mechanisms underlying PAW's disinfection effectiveness involve multifaceted oxidative assaults on microbial physiology. Primary long-lived species like H₂O₂ penetrate cell membranes via aquaporins, triggering Fenton-like reactions with intracellular iron to generate •OH radicals. These highly reactive hydroxyl species cause irreparable damage: lipid peroxidation disrupts membrane integrity, protein carbonylation inactivates essential enzymes, and DNA/RNA strand scission halts replication. Synergistic RONS, such as peroxynitrite (ONOO⁻) from NO• and O₂•⁻, exhibit enhanced potency, attacking thiol groups in proteins and disrupting quorum sensing in biofilms prevalent in aquaculture. The acidic pH (2.0–5.5) and elevated oxidation-reduction potential (ORP > +400 mV) amplify these effects by protonating membrane lipids, enhancing permeability and facilitating species influx. Nitrous acid (HNO₂), formed from NO₂⁻ protonation, inhibits respiratory cytochromes, suffocating aerobic pathogens.

Research highlights H₂O₂'s stability and antimicrobial potency in PAW, with concentrations maintained during storage under cool, dark conditions, enabling reliable field deployment (Vichiansan et al., 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5vcnJhcG9uIiwiZmFtaWx5IjoiVmljaGlhbnNhbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQb3Jua2FyYW4iLCJmYW1pbHkiOiJQaGFuLXVtb25nIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRoaXBwYXdhbiIsImZhbWlseSI6Ik1la3dpbGFpIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkphdHVyb25nIiwiZmFtaWx5IjoiS3VtbGEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiS29tZ3JpdCIsImZhbWlseSI6Ikxla3Nha3VsIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNhaXNhbW9ybiIsImZhbWlseSI6Ikx1bXlvbmcifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiRGhlZXJhd2FuIiwiZmFtaWx5IjoiQm9vbnlhd2FuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlBpcGF0aCIsImZhbWlseSI6IlBvcmFtYXBpaml0d2F0In19XSwiaWRzIjp7Imlzc24iOltdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMjEyMDMvcnMuMy5ycy0zMjMwNTYxL3YxIiwib3BlbmFsZXgiOiJXNDM4NTk1NDk1MyJ9LCJ0aXRsZSI6IkFwcHJvcHJpYXRlIFBhcmFtZXRlcnMgZm9yIGdyb3d0aCBSZWR1Y3Rpb24gb2YgbWljcm9vcmdhbmlzbXMgaW4gV2hpdGUgbGVnIFNocmltcCAoTGl0b3BlbmFldXMgdmFubmFtZWkpIGFuZCBTcGxlbmRpZCBTcXVpZCAoTG9saWdvIGZvcm1vc2FuYSkgYnkgUGxhc21hIEFjdGl2YXRlZCBXYXRlciIsImFic3RyYWN0IjoiQWJzdHJhY3QgVGhlIHN0dWR5IGZvY3VzZXMgb24gdGhlIGFwcGxpY2F0aW9uIG9mIFBBVyBpbmFjdGl2YXRpb24gZm9yIGV4cG9ydGluZyB3aGl0ZSBsZWcgc2hyaW1wIGFuZCBzcGxlbmRpZCBzcXVpZC4gVGhlIGV4cGVyaW1lbnQgZGV0ZXJtaW5lZCB0aGUgbWF4aW11bSBoeWRyb2dlbiBwZXJveGlkZSBjb25jZW50cmF0aW9uIGluIFBBVyBieSB1dGlsaXppbmcgYSBmdWxsIGZhY3RvcmlhbCBkZXNpZ24gd2l0aCB0d28tbGV2ZWwgZmFjdG9ycy4gU3Vic2VxdWVudGx5LCB0aGUgYW5hbHlzaXMgb2YgdmFyaWFuY2UgKEFOT1ZBKSB3YXMgY29uZHVjdGVkIHVzaW5nIE1pbml0YWIuIFRoZSByZXN1bHRzIGluZGljYXRlZCB0aGF0IGEgZ2FzIGZsb3cgcmF0ZSBvZiBhIG1peHR1cmUgb2YgQXIgKDk4JSkgYW5kIE8gMiAoMiUpIGF0IDI1IEwvbWluLCB3aXRoIGEgZGlzY2hhcmdlIHRpbWUgb2YgMTIwIG1pbnV0ZXMsIGdlbmVyYXRlZCB0aGUgb3B0aW1hbCBjb25jZW50cmF0aW9uIG9mIEggMiBPIDIgaW4gUEFXIGF0IDEzLjMwIG1nL0wsIGFzIHBlciB0aGUgcmVzcG9uc2Ugb3B0aW1pemVyLiBUaGUgdXNlIG9mIFBBVyB0byBpbmhpYml0IHRoZSBncm93dGggb2YgbWljcm9vcmdhbmlzbXMgaW4gd2hpdGUgbGVnIHNocmltcCBhbmQgc3BsZW5kaWQgc3F1aWQgZGVtb25zdHJhdGVkIHRoYXQgaW5hY3RpdmF0aW9uIHdpdGggUEFXLCBzb2FrZWQgZm9yIDEwIG1pbnV0ZXMsIHJlc3VsdGVkIGluIGEgcmVkdWN0aW9uIG9mIHRvdGFsIG1pY3Jvb3JnYW5pc21zIHRvIGxlc3MgdGhhbiA2IExvZyBDRlUvZy4gQWRkaXRpb25hbGx5LCBhbiBleGFtaW5hdGlvbiBvZiB0aGUgbnV0cml0aW9uIG9mIGFxdWFjdWx0dXJlIGZvb2QgcmV2ZWFsZWQgdGhhdCBQQVctaW1tZXJzZWQgd2hpdGUgbGVnIHNocmltcCBhbmQgc3BsZW5kaWQgc3F1aWQgc2hvd2VkIGRpZmZlcmVudCBlZmZlY3RzIG9uIHByb3RlaW4gYW5kIGxpcGlkIGNoYW5nZXMuIFRoZSByZXN1bHRzIHN1Z2dlc3QgdGhhdCBQQVcgY291bGQgYmUgYSB2aWFibGUgb3B0aW9uIGZvciBkZWNvbnRhbWluYXRpbmcgYXF1YXRpYyBmb29kIHByb2R1Y3RzLiBIb3dldmVyLCBpdCBpcyBpbXBvcnRhbnQgdG8gY29uc2lkZXIgdGhhdCBveGlkYXRpb24gbWF5IHBvc2UgYSBjaGFsbGVuZ2UgaW4gY2VydGFpbiBwcm9kdWN0cy4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjEyMDMvcnMuMy5ycy0zMjMwNTYxL3YxIiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5yZXNlYXJjaHNxdWFyZS5jb20vYXJ0aWNsZS9ycy0zMjMwNTYxL2xhdGVzdC5wZGYiLCJpc3N1ZWREYXRlIjoiMjAyMy0wOC0xNyIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDgiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IlJlc2VhcmNoIFNxdWFyZSAoUmVzZWFyY2ggU3F1YXJlKSIsImltcGFjdEZhY3RvciI6IjAuNDAifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiUmVzZWFyY2ggU3F1YXJlIChVbml0ZWQgU3RhdGVzKSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NDAsImNpdGVkQnlDb3VudCI6MH19LCJxdW90ZSI6Ikh5ZHJvZ2VuIHBlcm94aWRlIHByb2R1Y3Rpb246IE8rIEgyIE%2FihpIgT0jii4UgT0jii4UrT0jii4XihpJIMk8yICgzKSBOaXRyYXRlIHByb2R1Y3Rpb246IE4yICsgTzIg4oaSIDJOTyAoNCkgMk5PICsgTzIg4oaSIDJOTzIgKDUpIDNOTzIgKyBIMiBP4oaSIDJITk8zICsgTk8gKDYpIEgyTzIgaXMgYSByZWxhdGl2ZWx5IHN0YWJsZSBjb21wb3VuZCwgYW5kIGl0cyBiZWhhdmlvciBpbiBsaXF1aWQgZW52aXJvbm1lbnRzIGNhbiBsZWFkIHRvIHRoZSBmb3JtYXRpb24gb2YgaHlkcm9wZXJveHkgcmFkaWNhbHMgb3Igc3VwZXJveGlkZXMsIG9yIGV2ZW4gaHlkcm94eWwgcmFkaWNhbHMuIFJlc2VhcmNoIGNvbmR1Y3RlZCBieSBOaXF1ZXQgZXQgYWwuICgyMDE4KSB1c2luZyBhIGRpZWxlY3RyaWMgcGxhc21hIHN5c3RlbSBkZW1vbnN0cmF0ZWQgdGhhdCBIMk8yIGNvbmNlbnRyYXRpb25zIGluIFBBVyBpbmNyZWFzZWQgd2l0aCBsb25nZXIgcGxhc21hIHRyZWF0bWVudCB0aW1lcyBhbmQgcmVtYWluZWQgc3RhYmxlIGR1cmluZyBleHRlbmRlZCBzdG9yYWdlIHBlcmlvZHMuIEgyTzIgaXMga25vd24gZm9yIGl0cyBzaWduae%2B%2FvWNhbnQgYW50aW1pY3JvYmlhbCBhbmQgY3l0b3RveGljIHByb3BlcnRpZXMgaW4gcGxhc21hLWFjdGl2YXRlZCBsaXF1aWRzIChKdWzDoWssIEh1amFjb3bDoSBldCBhbC4sIDIwMTgpLiBUaGUgZXhwZXJpbWVudCByZXZlYWxlZCB0aGF0IHRoZSBkZWNvbXBvc2l0aW9uIHJhdGUgb2YgSDJPMiBpcyB0ZW1wZXJhdHVyZS1kZXBlbmRlbnQsIHdpdGggaGlnaGVyIHRlbXBlcmF0dXJlcyBsZWFkaW5nIHRvIGEgZGVjcmVhc2UgaW4gSDJPMiBjb25jZW50cmF0aW9uLiBDb252ZXJzZWx5LCBsb3dlciB0ZW1wZXJhdHVyZXMgZmFjaWxpdGF0ZSB0aGUgbWFpbnRlbmFuY2Ugb2YgSDJPMiBjb25jZW50cmF0aW9uIG92ZXIgYSBsb25nZXIgZHVyYXRpb24uIFRoaXMgaW5kaWNhdGVzIHRoYXQgc3RvcmluZyBQQVcgYXQgbG93IHRlbXBlcmF0dXJlcyBiZXR0ZXIgcHJlc2VydmVzIHRoZSBjb25jZW50cmF0aW9uIG9mIEgyTzIgY29tcGFyZWQgdG8gc3RvcmluZyBpdCBhdCBoaWdoIHRlbXBlcmF0dXJlcy4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6MjAuNDgzNjc1MDAzMDUxNzU4LCJsZWZ0Ijo0LjU3NTE2Mjg4NzU3MzI0Miwid2lkdGgiOjI2LjQ3OTAzODIzODUyNTM5LCJoZWlnaHQiOjEuNzc3MTYzMzg2MzQ0OTA5N319LHsicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjoyNC40MDE5MDEyNDUxMTcxODgsImxlZnQiOjQuNTc1MTYyODg3NTczMjQyLCJ3aWR0aCI6MzAuMjQzMTQxMTc0MzE2NDA2LCJoZWlnaHQiOjEuODM5Nzg2MDUyNzAzODU3NH19LHsicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjoyOC4yNTU3OTI2MTc3OTc4NSwibGVmdCI6NC41NzUxNjI4ODc1NzMyNDIsIndpZHRoIjoxNi4wNTMwMTI4NDc5MDAzOSwiaGVpZ2h0IjoxLjc3NzE2MzM4NjM0NDkwOTd9fSx7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6MzIuMDQ3MDYxOTIwMTY2MDE2LCJsZWZ0Ijo0LjU3NTE2Mjg4NzU3MzI0Miwid2lkdGgiOjI3LjY2NzI5OTI3MDYyOTg4MywiaGVpZ2h0Ijo2LjMyNjY4NzMzNTk2ODAxOH19LHsicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjo0MC4zODc5MjAzNzk2Mzg2NywibGVmdCI6NC41NzUxNjI4ODc1NzMyNDIsIndpZHRoIjo5MC43MTMxMTk1MDY4MzU5NCwiaGVpZ2h0IjoyLjA2MzcwMTE1MjgwMTUxMzd9fSx7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6NDIuODUyMTk1NzM5NzQ2MDk0LCJsZWZ0Ijo0LjU3NTE2Mjg4NzU3MzI0Miwid2lkdGgiOjg1LjM5NjYyMTcwNDEwMTU2LCJoZWlnaHQiOjEuNzc3MTYzMzg2MzQ0OTA5N319LHsicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjo0NS4xMjcwMjE3ODk1NTA3OCwibGVmdCI6NC41NzUxNjI4ODc1NzMyNDIsIndpZHRoIjo4OC43ODIyMTEzMDM3MTA5NCwiaGVpZ2h0IjoyLjA2MzcwMTE1MjgwMTUxMzd9fSx7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6NDcuNTkxMjk3MTQ5NjU4MiwibGVmdCI6NC41NzUxNjI4ODc1NzMyNDIsIndpZHRoIjo4OS40NjkyOTE2ODcwMTE3MiwiaGVpZ2h0IjoyLjA2Mzc2MjY2NDc5NDkyMn19LHsicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjo1MC4wNTU2NDExNzQzMTY0MDYsImxlZnQiOjQuNTc1MTYyODg3NTczMjQyLCJ3aWR0aCI6ODguNTc3MTQwODA4MTA1NDcsImhlaWdodCI6NC4zMzg1MjQzNDE1ODMyNTJ9fSx7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6NTQuNzk0NzM4NzY5NTMxMjUsImxlZnQiOjQuNTc1MTYyODg3NTczMjQyLCJ3aWR0aCI6ODMuMzE2MjA3ODg1NzQyMTksImhlaWdodCI6Mi4wNjM3NjI2NjQ3OTQ5MjJ9fSx7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6NTcuMjU5MDgyNzk0MTg5NDUsImxlZnQiOjQuNTc1MTYyODg3NTczMjQyLCJ3aWR0aCI6ODkuNTY1MTI0NTExNzE4NzUsImhlaWdodCI6Mi4wNjM3NjI2NjQ3OTQ5MjJ9fSx7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6NTkuNzIzNDI2ODE4ODQ3NjU2LCJsZWZ0Ijo0LjU3NTE2Mjg4NzU3MzI0Miwid2lkdGgiOjgwLjA3MzQxMDAzNDE3OTY5LCJoZWlnaHQiOjIuMDYzNzYyNjY0Nzk0OTIyfX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjYyLjE4Nzc2NzAyODgwODU5NCwibGVmdCI6NC41NzUxNjI4ODc1NzMyNDIsIndpZHRoIjoxMS45MjM4MDkwNTE1MTM2NzIsImhlaWdodCI6MS43NzcxNjMzODYzNDQ5MDk3fX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiMTEifX1d.

In practical aquaculture applications, PAW has been successfully integrated for water disinfection in hatcheries, grow-out ponds, and RAS. The major applications of PAW in aquaculture and their intended benefits are summarized in Table 4. Direct immersion or spraying of PAW into culture tanks rapidly curbs outbreaks of vibriosis and aeromoniasis, reducing total viable counts by 99.999% within 30–60 minutes. For example, treating shrimp culture water with PAW generated via dielectric barrier discharge plasma yielded significant microbial growth inhibition in Litopenaeus vannamei and Loligo formosana systems, correlating with improved survival rates (Vichiansan et al., 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5vcnJhcG9uIiwiZmFtaWx5IjoiVmljaGlhbnNhbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQb3Jua2FyYW4iLCJmYW1pbHkiOiJQaGFuLXVtb25nIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRoaXBwYXdhbiIsImZhbWlseSI6Ik1la3dpbGFpIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkphdHVyb25nIiwiZmFtaWx5IjoiS3VtbGEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiS29tZ3JpdCIsImZhbWlseSI6Ikxla3Nha3VsIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNhaXNhbW9ybiIsImZhbWlseSI6Ikx1bXlvbmcifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiRGhlZXJhd2FuIiwiZmFtaWx5IjoiQm9vbnlhd2FuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlBpcGF0aCIsImZhbWlseSI6IlBvcmFtYXBpaml0d2F0In19XSwiaWRzIjp7Imlzc24iOltdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMjEyMDMvcnMuMy5ycy0zMjMwNTYxL3YxIiwib3BlbmFsZXgiOiJXNDM4NTk1NDk1MyJ9LCJ0aXRsZSI6IkFwcHJvcHJpYXRlIFBhcmFtZXRlcnMgZm9yIGdyb3d0aCBSZWR1Y3Rpb24gb2YgbWljcm9vcmdhbmlzbXMgaW4gV2hpdGUgbGVnIFNocmltcCAoTGl0b3BlbmFldXMgdmFubmFtZWkpIGFuZCBTcGxlbmRpZCBTcXVpZCAoTG9saWdvIGZvcm1vc2FuYSkgYnkgUGxhc21hIEFjdGl2YXRlZCBXYXRlciIsImFic3RyYWN0IjoiQWJzdHJhY3QgVGhlIHN0dWR5IGZvY3VzZXMgb24gdGhlIGFwcGxpY2F0aW9uIG9mIFBBVyBpbmFjdGl2YXRpb24gZm9yIGV4cG9ydGluZyB3aGl0ZSBsZWcgc2hyaW1wIGFuZCBzcGxlbmRpZCBzcXVpZC4gVGhlIGV4cGVyaW1lbnQgZGV0ZXJtaW5lZCB0aGUgbWF4aW11bSBoeWRyb2dlbiBwZXJveGlkZSBjb25jZW50cmF0aW9uIGluIFBBVyBieSB1dGlsaXppbmcgYSBmdWxsIGZhY3RvcmlhbCBkZXNpZ24gd2l0aCB0d28tbGV2ZWwgZmFjdG9ycy4gU3Vic2VxdWVudGx5LCB0aGUgYW5hbHlzaXMgb2YgdmFyaWFuY2UgKEFOT1ZBKSB3YXMgY29uZHVjdGVkIHVzaW5nIE1pbml0YWIuIFRoZSByZXN1bHRzIGluZGljYXRlZCB0aGF0IGEgZ2FzIGZsb3cgcmF0ZSBvZiBhIG1peHR1cmUgb2YgQXIgKDk4JSkgYW5kIE8gMiAoMiUpIGF0IDI1IEwvbWluLCB3aXRoIGEgZGlzY2hhcmdlIHRpbWUgb2YgMTIwIG1pbnV0ZXMsIGdlbmVyYXRlZCB0aGUgb3B0aW1hbCBjb25jZW50cmF0aW9uIG9mIEggMiBPIDIgaW4gUEFXIGF0IDEzLjMwIG1nL0wsIGFzIHBlciB0aGUgcmVzcG9uc2Ugb3B0aW1pemVyLiBUaGUgdXNlIG9mIFBBVyB0byBpbmhpYml0IHRoZSBncm93dGggb2YgbWljcm9vcmdhbmlzbXMgaW4gd2hpdGUgbGVnIHNocmltcCBhbmQgc3BsZW5kaWQgc3F1aWQgZGVtb25zdHJhdGVkIHRoYXQgaW5hY3RpdmF0aW9uIHdpdGggUEFXLCBzb2FrZWQgZm9yIDEwIG1pbnV0ZXMsIHJlc3VsdGVkIGluIGEgcmVkdWN0aW9uIG9mIHRvdGFsIG1pY3Jvb3JnYW5pc21zIHRvIGxlc3MgdGhhbiA2IExvZyBDRlUvZy4gQWRkaXRpb25hbGx5LCBhbiBleGFtaW5hdGlvbiBvZiB0aGUgbnV0cml0aW9uIG9mIGFxdWFjdWx0dXJlIGZvb2QgcmV2ZWFsZWQgdGhhdCBQQVctaW1tZXJzZWQgd2hpdGUgbGVnIHNocmltcCBhbmQgc3BsZW5kaWQgc3F1aWQgc2hvd2VkIGRpZmZlcmVudCBlZmZlY3RzIG9uIHByb3RlaW4gYW5kIGxpcGlkIGNoYW5nZXMuIFRoZSByZXN1bHRzIHN1Z2dlc3QgdGhhdCBQQVcgY291bGQgYmUgYSB2aWFibGUgb3B0aW9uIGZvciBkZWNvbnRhbWluYXRpbmcgYXF1YXRpYyBmb29kIHByb2R1Y3RzLiBIb3dldmVyLCBpdCBpcyBpbXBvcnRhbnQgdG8gY29uc2lkZXIgdGhhdCBveGlkYXRpb24gbWF5IHBvc2UgYSBjaGFsbGVuZ2UgaW4gY2VydGFpbiBwcm9kdWN0cy4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjEyMDMvcnMuMy5ycy0zMjMwNTYxL3YxIiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5yZXNlYXJjaHNxdWFyZS5jb20vYXJ0aWNsZS9ycy0zMjMwNTYxL2xhdGVzdC5wZGYiLCJpc3N1ZWREYXRlIjoiMjAyMy0wOC0xNyIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDgiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IlJlc2VhcmNoIFNxdWFyZSAoUmVzZWFyY2ggU3F1YXJlKSIsImltcGFjdEZhY3RvciI6IjAuNDAifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiUmVzZWFyY2ggU3F1YXJlIChVbml0ZWQgU3RhdGVzKSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NDAsImNpdGVkQnlDb3VudCI6MH19LCJxdW90ZSI6Ikh5ZHJvZ2VuIHBlcm94aWRlIHByb2R1Y3Rpb246IE8rIEgyIE%2FihpIgT0jii4UgT0jii4UrT0jii4XihpJIMk8yICgzKSBOaXRyYXRlIHByb2R1Y3Rpb246IE4yICsgTzIg4oaSIDJOTyAoNCkgMk5PICsgTzIg4oaSIDJOTzIgKDUpIDNOTzIgKyBIMiBP4oaSIDJITk8zICsgTk8gKDYpIEgyTzIgaXMgYSByZWxhdGl2ZWx5IHN0YWJsZSBjb21wb3VuZCwgYW5kIGl0cyBiZWhhdmlvciBpbiBsaXF1aWQgZW52aXJvbm1lbnRzIGNhbiBsZWFkIHRvIHRoZSBmb3JtYXRpb24gb2YgaHlkcm9wZXJveHkgcmFkaWNhbHMgb3Igc3VwZXJveGlkZXMsIG9yIGV2ZW4gaHlkcm94eWwgcmFkaWNhbHMuIFJlc2VhcmNoIGNvbmR1Y3RlZCBieSBOaXF1ZXQgZXQgYWwuICgyMDE4KSB1c2luZyBhIGRpZWxlY3RyaWMgcGxhc21hIHN5c3RlbSBkZW1vbnN0cmF0ZWQgdGhhdCBIMk8yIGNvbmNlbnRyYXRpb25zIGluIFBBVyBpbmNyZWFzZWQgd2l0aCBsb25nZXIgcGxhc21hIHRyZWF0bWVudCB0aW1lcyBhbmQgcmVtYWluZWQgc3RhYmxlIGR1cmluZyBleHRlbmRlZCBzdG9yYWdlIHBlcmlvZHMuIEgyTzIgaXMga25vd24gZm9yIGl0cyBzaWduae%2B%2FvWNhbnQgYW50aW1pY3JvYmlhbCBhbmQgY3l0b3RveGljIHByb3BlcnRpZXMgaW4gcGxhc21hLWFjdGl2YXRlZCBsaXF1aWRzIChKdWzDoWssIEh1amFjb3bDoSBldCBhbC4sIDIwMTgpLiBUaGUgZXhwZXJpbWVudCByZXZlYWxlZCB0aGF0IHRoZSBkZWNvbXBvc2l0aW9uIHJhdGUgb2YgSDJPMiBpcyB0ZW1wZXJhdHVyZS1kZXBlbmRlbnQsIHdpdGggaGlnaGVyIHRlbXBlcmF0dXJlcyBsZWFkaW5nIHRvIGEgZGVjcmVhc2UgaW4gSDJPMiBjb25jZW50cmF0aW9uLiBDb252ZXJzZWx5LCBsb3dlciB0ZW1wZXJhdHVyZXMgZmFjaWxpdGF0ZSB0aGUgbWFpbnRlbmFuY2Ugb2YgSDJPMiBjb25jZW50cmF0aW9uIG92ZXIgYSBsb25nZXIgZHVyYXRpb24uIFRoaXMgaW5kaWNhdGVzIHRoYXQgc3RvcmluZyBQQVcgYXQgbG93IHRlbXBlcmF0dXJlcyBiZXR0ZXIgcHJlc2VydmVzIHRoZSBjb25jZW50cmF0aW9uIG9mIEgyTzIgY29tcGFyZWQgdG8gc3RvcmluZyBpdCBhdCBoaWdoIHRlbXBlcmF0dXJlcy4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6MjAuNDgzNjc1MDAzMDUxNzU4LCJsZWZ0Ijo0LjU3NTE2Mjg4NzU3MzI0Miwid2lkdGgiOjI2LjQ3OTAzODIzODUyNTM5LCJoZWlnaHQiOjEuNzc3MTYzMzg2MzQ0OTA5N319LHsicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjoyNC40MDE5MDEyNDUxMTcxODgsImxlZnQiOjQuNTc1MTYyODg3NTczMjQyLCJ3aWR0aCI6MzAuMjQzMTQxMTc0MzE2NDA2LCJoZWlnaHQiOjEuODM5Nzg2MDUyNzAzODU3NH19LHsicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjoyOC4yNTU3OTI2MTc3OTc4NSwibGVmdCI6NC41NzUxNjI4ODc1NzMyNDIsIndpZHRoIjoxNi4wNTMwMTI4NDc5MDAzOSwiaGVpZ2h0IjoxLjc3NzE2MzM4NjM0NDkwOTd9fSx7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6MzIuMDQ3MDYxOTIwMTY2MDE2LCJsZWZ0Ijo0LjU3NTE2Mjg4NzU3MzI0Miwid2lkdGgiOjI3LjY2NzI5OTI3MDYyOTg4MywiaGVpZ2h0Ijo2LjMyNjY4NzMzNTk2ODAxOH19LHsicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjo0MC4zODc5MjAzNzk2Mzg2NywibGVmdCI6NC41NzUxNjI4ODc1NzMyNDIsIndpZHRoIjo5MC43MTMxMTk1MDY4MzU5NCwiaGVpZ2h0IjoyLjA2MzcwMTE1MjgwMTUxMzd9fSx7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6NDIuODUyMTk1NzM5NzQ2MDk0LCJsZWZ0Ijo0LjU3NTE2Mjg4NzU3MzI0Miwid2lkdGgiOjg1LjM5NjYyMTcwNDEwMTU2LCJoZWlnaHQiOjEuNzc3MTYzMzg2MzQ0OTA5N319LHsicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjo0NS4xMjcwMjE3ODk1NTA3OCwibGVmdCI6NC41NzUxNjI4ODc1NzMyNDIsIndpZHRoIjo4OC43ODIyMTEzMDM3MTA5NCwiaGVpZ2h0IjoyLjA2MzcwMTE1MjgwMTUxMzd9fSx7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6NDcuNTkxMjk3MTQ5NjU4MiwibGVmdCI6NC41NzUxNjI4ODc1NzMyNDIsIndpZHRoIjo4OS40NjkyOTE2ODcwMTE3MiwiaGVpZ2h0IjoyLjA2Mzc2MjY2NDc5NDkyMn19LHsicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjo1MC4wNTU2NDExNzQzMTY0MDYsImxlZnQiOjQuNTc1MTYyODg3NTczMjQyLCJ3aWR0aCI6ODguNTc3MTQwODA4MTA1NDcsImhlaWdodCI6NC4zMzg1MjQzNDE1ODMyNTJ9fSx7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6NTQuNzk0NzM4NzY5NTMxMjUsImxlZnQiOjQuNTc1MTYyODg3NTczMjQyLCJ3aWR0aCI6ODMuMzE2MjA3ODg1NzQyMTksImhlaWdodCI6Mi4wNjM3NjI2NjQ3OTQ5MjJ9fSx7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6NTcuMjU5MDgyNzk0MTg5NDUsImxlZnQiOjQuNTc1MTYyODg3NTczMjQyLCJ3aWR0aCI6ODkuNTY1MTI0NTExNzE4NzUsImhlaWdodCI6Mi4wNjM3NjI2NjQ3OTQ5MjJ9fSx7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6NTkuNzIzNDI2ODE4ODQ3NjU2LCJsZWZ0Ijo0LjU3NTE2Mjg4NzU3MzI0Miwid2lkdGgiOjgwLjA3MzQxMDAzNDE3OTY5LCJoZWlnaHQiOjIuMDYzNzYyNjY0Nzk0OTIyfX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjYyLjE4Nzc2NzAyODgwODU5NCwibGVmdCI6NC41NzUxNjI4ODc1NzMyNDIsIndpZHRoIjoxMS45MjM4MDkwNTE1MTM2NzIsImhlaWdodCI6MS43NzcxNjMzODYzNDQ5MDk3fX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiMTEifX1d. Biofloc systems benefit from periodic PAW dosing to control opportunistic pathogens without disrupting beneficial microbiota. Unlike UV irradiation or ozonation, which require clear water and energy-intensive equipment, PAW operates in turbid conditions and scales easily via portable plasma generators. Field trials report 20–50% reductions in antibiotic use, mitigating resistance risks while enhancing water clarity through flocculated debris.

Table 4: Applications of plasma-activated water in aquaculture

| Application | Mechanism | Benefits |
| --- | --- | --- |
| Water disinfection | Inactivation of microorganisms through reactive species | Reduced microbial load, improved water quality |
| Fish health and immunity | Stimulation of immune system, promotion of wound healing | Enhanced disease resistance, improved survival rates |
| Growth promotion | Stimulation of microalgal growth, enhanced feed conversion | Increased growth performance, natural food availability |

PAW's disinfection extends to viral and fungal threats, such as white spot syndrome virus in shrimp and Saprolegnia in salmonids. Viral capsids are denatured by •OH-induced protein oxidation, while fungal hyphae succumb to cell wall chitin degradation under prolonged RONS exposure. Integration with complementary technologies amplifies efficacy; combining PAW with oxygen nanobubbles sustains high dissolved oxygen levels, countering hypoxia from organic decay and boosting aerobic antagonist microbes (Tekile et al., 2016)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFuZGluZXQgS2ViZWRlIiwiZmFtaWx5IjoiVGVraWxlIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikls4oCQSG8iLCJmYW1pbHkiOiJLaW0ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSmFpeWVvcCIsImZhbWlseSI6IkxlZSJ9fV0sImlkcyI6eyJpc3NuIjpbIjEyMjYtMTAyNSIsIjIwMDUtOTY4WCJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuNDQ5MS9lZXIuMjAxNi4wMjgiLCJtYWciOiIyNTIzNjY5NjU3Iiwib3BlbmFsZXgiOiJXMjUyMzY2OTY1NyJ9LCJ0aXRsZSI6IkV4dGVudCBhbmQgcGVyc2lzdGVuY2Ugb2YgZGlzc29sdmVkIG94eWdlbiBlbmhhbmNlbWVudCB1c2luZyBuYW5vYnViYmxlcyIsImFic3RyYWN0IjoiSW4gdGhpcyBzdHVkeSwgY2hhbmdlIGluIHdhdGVyLWRpc3NvbHZlZCBveHlnZW4gKERPKSB3YXMgYW5hbHl6ZWQgdW5kZXIgdmFyaW91cyBzeW50aGV0aWMgd2F0ZXIgcXVhbGl0aWVzIGFuZCBuYW5vYnViYmxlcyAoTkJzKSBhcHBsaWNhdGlvbiBjb25kaXRpb25zLCBzdWNoIGFzIGdhcyB0eXBlLCBpbml0aWFsIERPIGFzIHdlbGwgYXMgd2F0ZXIgZGlzc29sdmVkLCBzdXNwZW5kZWQgYW5kIG9yZ2FuaWMgbWF0dGVycyBjb250ZW50cy4gV2hlbiBveHlnZW4sIHJhdGhlciB0aGFuIGFpciwgd2FzIGludHJvZHVjZWQgaW50byBuaXRyb2dlbi1kZXNvcmJlZCB1bHRyYS1wdXJlIHdhdGVyLCB0aGUgc3RhZ25hdGlvbiB0aW1lIHdhcyBzaWduaWZpY2FudGx5IGluY3JlYXNlZC4gSXQgdG9vayB0ZW4gZGF5cyBmb3IgRE8gY29uY2VudHJhdGlvbiB0byBkcm9wIGJhY2sgdG8gc2F0dXJhdGlvbi4gVGhlIGhpZ2hlciB0aGUgaW5pdGlhbCBETyBjb25jZW50cmF0aW9uLCB0aGUgbG9uZ2VyIHBhcnRpY2xlcyB3ZXJlIG9ic2VydmVkIGFib3ZlIHNhdHVyYXRpb24gZHVlIHRvIHBhcnRpY2xlIHN0YWJpbGl0eSBpbXByb3ZlbWVudC4gVGhlIG94eWdlbiBtYXNzIHRyYW5zZmVyIHJhdGUgb2YgMC4wNDgyIG1nL0wvbWluIHdhcyBmb3VuZCB0byByZWFjaCBhIG1heGltdW0gYXQgYW4gZWxlY3Ryb2x5dGljIGNvbmNlbnRyYXRpb24gb2YgMC43NSBnL0wsIGJleW9uZCB3aGljaCB0aGUgdHJhbnNmZXIgcmF0ZSBkZWNyZWFzZWQgZHVlIHRvIGFkc29ycHRpb24gb2YgbmVnYXRpdmUgaW9ucyBvZiB0aGUgZWxlY3Ryb2x5dGUgYXQgdGhlIGludGVyZmFjZS4gSGlnaCBsZXZlbHMgb2YgdHVyYmlkaXR5IGNhdXNlZCBieSBzdXNwZW5kZWQgc29saWRzIGhhdmUgYmVjb21lIGEgYmFycmllciB0byBkaXNzb2x1dGlvbiBvZiBOQnMgb3h5Z2VuIGludG8gdGhlIHdhdGVyIHNvbHV0aW9uLCBhbmQgdGh1cyBhZmZlY3RlZCB0aGUgdHJhbnNmZXIgcGVyZm9ybWFuY2UuIE9uIHRoZSBvdGhlciBoYW5kLCBieSBhcHBseWluZyBOQnMgZm9yIGp1c3QgYW4gaG91ciwgdXAgdG8gNy4yJSBkZWdyYWRhdGlvbiBvZiBnbHVjb3NlIGFzIHJlcHJlc2VudGF0aXZlIG9yZ2FuaWMgbWF0dGVyIHdhcyBhY2hpZXZlZC4gVGh1cywgTkJzIHRlY2hub2xvZ3kgd291bGQgbWFpbnRhaW4gYSBoaWdoIERPIGV4dGVudCBmb3IgYW4gZXh0ZW5kZWQgZHVyYXRpb24sIGFuZCB0aHVzIGNhbiBpbXByb3ZlIHdhdGVyIHF1YWxpdHkgcHJvdmlkZWQgdGhhdCB3YXRlciBjaGVtaXN0cnkgaXMgY2xvc2VseSBtb25pdG9yZWQgZHVyaW5nIGl0cyBhcHBsaWNhdGlvbi4gS2V5d29yZHM6IEFwcGxpY2F0aW9uIGNvbmRpdGlvbnMsIERpc3NvbHZlZCBveHlnZW4sIE5hbm9idWJibGVzLCBXYXRlciBxdWFsaXR5IiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjQ0OTEvZWVyLjIwMTYuMDI4IiwicmVzb3VyY2VVcmwiOiJodHRwOi8vZWVlci5vcmcvdXBsb2FkL2Vlci0yMS00LTQyNy5wZGYiLCJpc3N1ZWREYXRlIjoiMjAxNi0wOS0xMiIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkVudmlyb25tZW50YWwgRW5naW5lZXJpbmcgUmVzZWFyY2giLCJpc3N1ZSI6NCwidm9sdW1lIjoyMSwiaW1wYWN0RmFjdG9yIjoiNC4wNSJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjQyNywibGFzdFBhZ2UiOjQzNX19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJLb3JlYW4gU29jaWV0eSBvZiBFbnZpcm9ubWVudGFsIEVuZ2luZWVyaW5nIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjoyNCwiY2l0ZWRCeUNvdW50IjozM319LCJxdW90ZSI6IlRoaXMgdmVyaWZpZXMgdGhlIHBvc3NpYmlsaXR5IG9mIHRoZSBleGlzdGVuY2UgIG9mIE5Ccywgc2luY2UgYXQgYSBoaWdoZXIgRE8gY29uY2VudHJhdGlvbiwgdGhlIG94eWdlbiBkaWZmdXNpb24gIHdvdWxkIHRha2UgbW9yZSB0aW1lLiBUaGUgaGlnaCBnYXMgY29uY2VudHJhdGlvbiBpbiB0aGUgbGlxdWlkICBleHBsYWlucyB0aGUgZXh0ZW5zaW9uIG9mIE5CIHN0YWJpbGl0eS4gIFRoZSBtYWduaXR1ZGVzIG9mIHpldGEgcG90ZW50aWFsIG9mIG94eWdlbiBuYW5vLWJ1YmJsZWQgd2F0ZXIgIHdlcmUgcG9zc2libHkgaGlnaGVyIHRoYW4gdGhvc2Ugb2YgYWlyIG5hbm8tYnViYmxlZCB3YXRlci4gQXQgIGEgaGlnaCBtYWduaXR1ZGUgemV0YSBwb3RlbnRpYWwgdmFsdWUsIHRoZSBlbGVjdHJpY2FsbHkgY2hhcmdlZCAgcGFydGljbGVzIHRlbmQgdG8gcmVwZWwgZWFjaCBvdGhlciwgYXZvaWRpbmcgYWdncmVnYXRpb24uIEZvciBidWJibGUgIGRpc3BlcnNpb24sIHRoZSBoaWdoIHpldGEgcG90ZW50aWFsIGNvdWxkIGNyZWF0ZSByZXB1bHNpb24gZm9yY2VzICB0aGF0IHdvdWxkIGF2b2lkIHRoZSBjb2FsZXNjZW5jZSBvZiBidWJibGVzLCB0aHVzIGNvbnRyaWJ1dGluZyAgdG8gdGhlIHN0YWJpbGl6YXRpb24gb2YgdGhlIGJ1YmJsZXMuIFRoZXJlZm9yZSwgdGhlIG94eWdlbiBOQnMgIGNvdWxkIGJlIHN0YWJpbGl6ZWQgaW4gd2F0ZXIgYnkgdGhlIGVsZWN0cmljYWxseSBjaGFyZ2VkIHN1cmZhY2VzLCAgd2hpbGUgdGhlIG5hbm8tYnViYmxlZCBhaXIgd291bGQgYmUgYmVsb3cgdGhlIGxpbWl0IG9mIHRoZSBzdGFiaWxpemEtdGlvbiBieSByZXB1bHNpb24gZm9yY2VzLiBGdXJ0aGVybW9yZSwgdGhlIGJpZyBkaWZmZXJlbmNlIGJldHdlZW4gdGhlIHBlcmlvZHMgaW4gd2hpY2ggIGJ1YmJsZXMgY291bGQgYmUgb2JzZXJ2ZWQgYXMgc3RhYmxlIGluIHRoZSBveHlnZW4gbmFuby1idWJibGVkICB3YXRlciAoMTAtMTUgZCkgYW5kIGluIHRoYXQgb2YgYWlyIG5hbm8tYnViYmxlZCAoMy01IGQpIGlzIGxpa2VseSBGaWcuIDYuIEdyYXBocyBzaG93aW5nIHN0YWduYXRpb24gdGltZSBmb3IgZGlmZmVyZW50IGluaXRpYWwgZGlzc29sdmVkIG94eWdlbiAoRE8pLiByZWxhdGVkIHRvIHRoZSBsb3dlciBkaXNzb2x1dGlvbiBvZiB0aGUgYWlyIGluIHdhdGVyIGluIHJlbGF0aW9uIHRvIHRoZSBkaXNzb2x1dGlvbiBvZiBveHlnZW4uICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6NjYuMTc4ODAyNDkwMjM0MzgsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6NDAuNzE0ODg5NTI2MzY3MTksImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo2Ny41NzY3NzQ1OTcxNjc5NywibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42ODgyNzA1Njg4NDc2NTYsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo2OC45NjcyMzE3NTA0ODgyOCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NTcxODA3ODYxMzI4MSwiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjcwLjM1NzY4ODkwMzgwODYsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6MjUuMzQxMjU1MTg3OTg4MjgsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3MS43NTU2NTMzODEzNDc2NiwibGVmdCI6OS41NDk2NDE2MDkxOTE4OTUsIndpZHRoIjozOS4wMjQ3ODQwODgxMzQ3NjYsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3My4xNDYxMDI5MDUyNzM0NCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NTQwMjk4NDYxOTE0MDYsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3NC41MzY1NjAwNTg1OTM3NSwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NTM0NjUyNzA5OTYwOTQsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3NS45MzQ1MzIxNjU1MjczNCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NzMyMzY4NDY5MjM4MywiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjc3LjMyNDk4OTMxODg0NzY2LCJsZWZ0Ijo3Ljg2NTU0NTI3MjgyNzE0OCwid2lkdGgiOjQwLjY0OTM1MzAyNzM0Mzc1LCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6NzguNzE1NDQ2NDcyMTY3OTcsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6NDAuNjU0NjI0OTM4OTY0ODQ0LCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODAuMTEzNDEwOTQ5NzA3MDMsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6NDAuNjUxMDUwNTY3NjI2OTUsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo4MS41MDM4NjA0NzM2MzI4MSwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NTQzNDI2NTEzNjcxOSwiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjgyLjg5NDMxNzYyNjk1MzEyLCJsZWZ0Ijo3Ljg2NTU0NTI3MjgyNzE0OCwid2lkdGgiOjQwLjMyMTgyMzEyMDExNzE5LCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODQuMjkyMjg5NzMzODg2NzIsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6MTUuODcwMjk1NTI0NTk3MTY4LCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODUuNjgyNzQ2ODg3MjA3MDMsImxlZnQiOjkuNTQ5NjQxNjA5MTkxODk1LCJ3aWR0aCI6MzguOTg5NzA0MTMyMDgwMDgsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo4Ny4wNzMyMDQwNDA1MjczNCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC43MTg0OTQ0MTUyODMyLCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODguNDcxMTY4NTE4MDY2NCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42ODc0MTk4OTEzNTc0MiwiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjM0LjUxMzI3MTMzMTc4NzExLCJsZWZ0Ijo1MS45MzI3Nzc0MDQ3ODUxNTYsIndpZHRoIjo0MC42NDcwNTI3NjQ4OTI1OCwiaGVpZ2h0IjoyLjcxODY2NjU1MzQ5NzMxNDV9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjozOC43MjUxNTg2OTE0MDYyNSwibGVmdCI6NTEuOTMyNzc3NDA0Nzg1MTU2LCJ3aWR0aCI6NDAuNjQzMTM1MDcwODAwNzgsImhlaWdodCI6MS4yNTMyODg1MDc0NjE1NDc5fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6NDAuMTE1NjEyMDMwMDI5MywibGVmdCI6NTEuOTMyNzc3NDA0Nzg1MTU2LCJ3aWR0aCI6NDAuNjY0NTg1MTEzNTI1MzksImhlaWdodCI6MS4yNTMyODg1MDc0NjE1NDc5fX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNDMxIn19XQ%3D%3D. This synergy optimizes RAS performance, as nanobubbles prolong DO elevation (10–15 days stability) via zeta potential repulsion, preventing pathogen resurgence (Tekile et al., 2016)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFuZGluZXQgS2ViZWRlIiwiZmFtaWx5IjoiVGVraWxlIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikls4oCQSG8iLCJmYW1pbHkiOiJLaW0ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSmFpeWVvcCIsImZhbWlseSI6IkxlZSJ9fV0sImlkcyI6eyJpc3NuIjpbIjEyMjYtMTAyNSIsIjIwMDUtOTY4WCJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuNDQ5MS9lZXIuMjAxNi4wMjgiLCJtYWciOiIyNTIzNjY5NjU3Iiwib3BlbmFsZXgiOiJXMjUyMzY2OTY1NyJ9LCJ0aXRsZSI6IkV4dGVudCBhbmQgcGVyc2lzdGVuY2Ugb2YgZGlzc29sdmVkIG94eWdlbiBlbmhhbmNlbWVudCB1c2luZyBuYW5vYnViYmxlcyIsImFic3RyYWN0IjoiSW4gdGhpcyBzdHVkeSwgY2hhbmdlIGluIHdhdGVyLWRpc3NvbHZlZCBveHlnZW4gKERPKSB3YXMgYW5hbHl6ZWQgdW5kZXIgdmFyaW91cyBzeW50aGV0aWMgd2F0ZXIgcXVhbGl0aWVzIGFuZCBuYW5vYnViYmxlcyAoTkJzKSBhcHBsaWNhdGlvbiBjb25kaXRpb25zLCBzdWNoIGFzIGdhcyB0eXBlLCBpbml0aWFsIERPIGFzIHdlbGwgYXMgd2F0ZXIgZGlzc29sdmVkLCBzdXNwZW5kZWQgYW5kIG9yZ2FuaWMgbWF0dGVycyBjb250ZW50cy4gV2hlbiBveHlnZW4sIHJhdGhlciB0aGFuIGFpciwgd2FzIGludHJvZHVjZWQgaW50byBuaXRyb2dlbi1kZXNvcmJlZCB1bHRyYS1wdXJlIHdhdGVyLCB0aGUgc3RhZ25hdGlvbiB0aW1lIHdhcyBzaWduaWZpY2FudGx5IGluY3JlYXNlZC4gSXQgdG9vayB0ZW4gZGF5cyBmb3IgRE8gY29uY2VudHJhdGlvbiB0byBkcm9wIGJhY2sgdG8gc2F0dXJhdGlvbi4gVGhlIGhpZ2hlciB0aGUgaW5pdGlhbCBETyBjb25jZW50cmF0aW9uLCB0aGUgbG9uZ2VyIHBhcnRpY2xlcyB3ZXJlIG9ic2VydmVkIGFib3ZlIHNhdHVyYXRpb24gZHVlIHRvIHBhcnRpY2xlIHN0YWJpbGl0eSBpbXByb3ZlbWVudC4gVGhlIG94eWdlbiBtYXNzIHRyYW5zZmVyIHJhdGUgb2YgMC4wNDgyIG1nL0wvbWluIHdhcyBmb3VuZCB0byByZWFjaCBhIG1heGltdW0gYXQgYW4gZWxlY3Ryb2x5dGljIGNvbmNlbnRyYXRpb24gb2YgMC43NSBnL0wsIGJleW9uZCB3aGljaCB0aGUgdHJhbnNmZXIgcmF0ZSBkZWNyZWFzZWQgZHVlIHRvIGFkc29ycHRpb24gb2YgbmVnYXRpdmUgaW9ucyBvZiB0aGUgZWxlY3Ryb2x5dGUgYXQgdGhlIGludGVyZmFjZS4gSGlnaCBsZXZlbHMgb2YgdHVyYmlkaXR5IGNhdXNlZCBieSBzdXNwZW5kZWQgc29saWRzIGhhdmUgYmVjb21lIGEgYmFycmllciB0byBkaXNzb2x1dGlvbiBvZiBOQnMgb3h5Z2VuIGludG8gdGhlIHdhdGVyIHNvbHV0aW9uLCBhbmQgdGh1cyBhZmZlY3RlZCB0aGUgdHJhbnNmZXIgcGVyZm9ybWFuY2UuIE9uIHRoZSBvdGhlciBoYW5kLCBieSBhcHBseWluZyBOQnMgZm9yIGp1c3QgYW4gaG91ciwgdXAgdG8gNy4yJSBkZWdyYWRhdGlvbiBvZiBnbHVjb3NlIGFzIHJlcHJlc2VudGF0aXZlIG9yZ2FuaWMgbWF0dGVyIHdhcyBhY2hpZXZlZC4gVGh1cywgTkJzIHRlY2hub2xvZ3kgd291bGQgbWFpbnRhaW4gYSBoaWdoIERPIGV4dGVudCBmb3IgYW4gZXh0ZW5kZWQgZHVyYXRpb24sIGFuZCB0aHVzIGNhbiBpbXByb3ZlIHdhdGVyIHF1YWxpdHkgcHJvdmlkZWQgdGhhdCB3YXRlciBjaGVtaXN0cnkgaXMgY2xvc2VseSBtb25pdG9yZWQgZHVyaW5nIGl0cyBhcHBsaWNhdGlvbi4gS2V5d29yZHM6IEFwcGxpY2F0aW9uIGNvbmRpdGlvbnMsIERpc3NvbHZlZCBveHlnZW4sIE5hbm9idWJibGVzLCBXYXRlciBxdWFsaXR5IiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjQ0OTEvZWVyLjIwMTYuMDI4IiwicmVzb3VyY2VVcmwiOiJodHRwOi8vZWVlci5vcmcvdXBsb2FkL2Vlci0yMS00LTQyNy5wZGYiLCJpc3N1ZWREYXRlIjoiMjAxNi0wOS0xMiIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkVudmlyb25tZW50YWwgRW5naW5lZXJpbmcgUmVzZWFyY2giLCJpc3N1ZSI6NCwidm9sdW1lIjoyMSwiaW1wYWN0RmFjdG9yIjoiNC4wNSJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjQyNywibGFzdFBhZ2UiOjQzNX19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJLb3JlYW4gU29jaWV0eSBvZiBFbnZpcm9ubWVudGFsIEVuZ2luZWVyaW5nIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjoyNCwiY2l0ZWRCeUNvdW50IjozM319LCJxdW90ZSI6IlRoaXMgdmVyaWZpZXMgdGhlIHBvc3NpYmlsaXR5IG9mIHRoZSBleGlzdGVuY2UgIG9mIE5Ccywgc2luY2UgYXQgYSBoaWdoZXIgRE8gY29uY2VudHJhdGlvbiwgdGhlIG94eWdlbiBkaWZmdXNpb24gIHdvdWxkIHRha2UgbW9yZSB0aW1lLiBUaGUgaGlnaCBnYXMgY29uY2VudHJhdGlvbiBpbiB0aGUgbGlxdWlkICBleHBsYWlucyB0aGUgZXh0ZW5zaW9uIG9mIE5CIHN0YWJpbGl0eS4gIFRoZSBtYWduaXR1ZGVzIG9mIHpldGEgcG90ZW50aWFsIG9mIG94eWdlbiBuYW5vLWJ1YmJsZWQgd2F0ZXIgIHdlcmUgcG9zc2libHkgaGlnaGVyIHRoYW4gdGhvc2Ugb2YgYWlyIG5hbm8tYnViYmxlZCB3YXRlci4gQXQgIGEgaGlnaCBtYWduaXR1ZGUgemV0YSBwb3RlbnRpYWwgdmFsdWUsIHRoZSBlbGVjdHJpY2FsbHkgY2hhcmdlZCAgcGFydGljbGVzIHRlbmQgdG8gcmVwZWwgZWFjaCBvdGhlciwgYXZvaWRpbmcgYWdncmVnYXRpb24uIEZvciBidWJibGUgIGRpc3BlcnNpb24sIHRoZSBoaWdoIHpldGEgcG90ZW50aWFsIGNvdWxkIGNyZWF0ZSByZXB1bHNpb24gZm9yY2VzICB0aGF0IHdvdWxkIGF2b2lkIHRoZSBjb2FsZXNjZW5jZSBvZiBidWJibGVzLCB0aHVzIGNvbnRyaWJ1dGluZyAgdG8gdGhlIHN0YWJpbGl6YXRpb24gb2YgdGhlIGJ1YmJsZXMuIFRoZXJlZm9yZSwgdGhlIG94eWdlbiBOQnMgIGNvdWxkIGJlIHN0YWJpbGl6ZWQgaW4gd2F0ZXIgYnkgdGhlIGVsZWN0cmljYWxseSBjaGFyZ2VkIHN1cmZhY2VzLCAgd2hpbGUgdGhlIG5hbm8tYnViYmxlZCBhaXIgd291bGQgYmUgYmVsb3cgdGhlIGxpbWl0IG9mIHRoZSBzdGFiaWxpemEtdGlvbiBieSByZXB1bHNpb24gZm9yY2VzLiBGdXJ0aGVybW9yZSwgdGhlIGJpZyBkaWZmZXJlbmNlIGJldHdlZW4gdGhlIHBlcmlvZHMgaW4gd2hpY2ggIGJ1YmJsZXMgY291bGQgYmUgb2JzZXJ2ZWQgYXMgc3RhYmxlIGluIHRoZSBveHlnZW4gbmFuby1idWJibGVkICB3YXRlciAoMTAtMTUgZCkgYW5kIGluIHRoYXQgb2YgYWlyIG5hbm8tYnViYmxlZCAoMy01IGQpIGlzIGxpa2VseSBGaWcuIDYuIEdyYXBocyBzaG93aW5nIHN0YWduYXRpb24gdGltZSBmb3IgZGlmZmVyZW50IGluaXRpYWwgZGlzc29sdmVkIG94eWdlbiAoRE8pLiByZWxhdGVkIHRvIHRoZSBsb3dlciBkaXNzb2x1dGlvbiBvZiB0aGUgYWlyIGluIHdhdGVyIGluIHJlbGF0aW9uIHRvIHRoZSBkaXNzb2x1dGlvbiBvZiBveHlnZW4uICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6NjYuMTc4ODAyNDkwMjM0MzgsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6NDAuNzE0ODg5NTI2MzY3MTksImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo2Ny41NzY3NzQ1OTcxNjc5NywibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42ODgyNzA1Njg4NDc2NTYsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo2OC45NjcyMzE3NTA0ODgyOCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NTcxODA3ODYxMzI4MSwiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjcwLjM1NzY4ODkwMzgwODYsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6MjUuMzQxMjU1MTg3OTg4MjgsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3MS43NTU2NTMzODEzNDc2NiwibGVmdCI6OS41NDk2NDE2MDkxOTE4OTUsIndpZHRoIjozOS4wMjQ3ODQwODgxMzQ3NjYsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3My4xNDYxMDI5MDUyNzM0NCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NTQwMjk4NDYxOTE0MDYsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3NC41MzY1NjAwNTg1OTM3NSwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NTM0NjUyNzA5OTYwOTQsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3NS45MzQ1MzIxNjU1MjczNCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NzMyMzY4NDY5MjM4MywiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjc3LjMyNDk4OTMxODg0NzY2LCJsZWZ0Ijo3Ljg2NTU0NTI3MjgyNzE0OCwid2lkdGgiOjQwLjY0OTM1MzAyNzM0Mzc1LCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6NzguNzE1NDQ2NDcyMTY3OTcsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6NDAuNjU0NjI0OTM4OTY0ODQ0LCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODAuMTEzNDEwOTQ5NzA3MDMsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6NDAuNjUxMDUwNTY3NjI2OTUsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo4MS41MDM4NjA0NzM2MzI4MSwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NTQzNDI2NTEzNjcxOSwiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjgyLjg5NDMxNzYyNjk1MzEyLCJsZWZ0Ijo3Ljg2NTU0NTI3MjgyNzE0OCwid2lkdGgiOjQwLjMyMTgyMzEyMDExNzE5LCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODQuMjkyMjg5NzMzODg2NzIsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6MTUuODcwMjk1NTI0NTk3MTY4LCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODUuNjgyNzQ2ODg3MjA3MDMsImxlZnQiOjkuNTQ5NjQxNjA5MTkxODk1LCJ3aWR0aCI6MzguOTg5NzA0MTMyMDgwMDgsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo4Ny4wNzMyMDQwNDA1MjczNCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC43MTg0OTQ0MTUyODMyLCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODguNDcxMTY4NTE4MDY2NCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42ODc0MTk4OTEzNTc0MiwiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjM0LjUxMzI3MTMzMTc4NzExLCJsZWZ0Ijo1MS45MzI3Nzc0MDQ3ODUxNTYsIndpZHRoIjo0MC42NDcwNTI3NjQ4OTI1OCwiaGVpZ2h0IjoyLjcxODY2NjU1MzQ5NzMxNDV9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjozOC43MjUxNTg2OTE0MDYyNSwibGVmdCI6NTEuOTMyNzc3NDA0Nzg1MTU2LCJ3aWR0aCI6NDAuNjQzMTM1MDcwODAwNzgsImhlaWdodCI6MS4yNTMyODg1MDc0NjE1NDc5fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6NDAuMTE1NjEyMDMwMDI5MywibGVmdCI6NTEuOTMyNzc3NDA0Nzg1MTU2LCJ3aWR0aCI6NDAuNjY0NTg1MTEzNTI1MzksImhlaWdodCI6MS4yNTMyODg1MDc0NjE1NDc5fX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNDMxIn19XQ%3D%3D.

Despite these advantages, optimal dosing remains paramount: real-time ORP/pH monitoring prevents over-oxidation that could stress filter feeders. Ongoing challenges include standardizing generation protocols across plasma devices and long-term ecotoxicity assessments. Nonetheless, PAW's residue-free, multi-target disinfection heralds a paradigm shift toward sustainable aquaculture water management, paving the way for chemical-free production. Beyond water quality management, the application of PAW extends directly to enhancing the physiological resilience of cultured species.

3.3.2. Fish Health and Immunity

Plasma-activated water (PAW) holds substantial promise for bolstering fish health and immunity in aquaculture settings, leveraging its rich profile of reactive oxygen and nitrogen species to modulate physiological responses without residual toxicity. Unlike traditional disinfectants, PAW's long-lived components, such as hydrogen peroxide (H₂O₂), nitrite (NO₂⁻), and nitrate (NO₃⁻), interact dynamically with fish physiology to prime immune defenses. Studies indicate that exposure to dilute PAW enhances innate immunity by upregulating antioxidant enzymes like superoxide dismutase and catalase, mitigating oxidative stress caused by environmental stressors or pathogens. This adaptive response increases disease resistance, as evidenced by higher survival rates in PAW-treated fish during challenge trials with bacterial pathogens like Aeromonas hydrophila. For instance, juvenile tilapia immersed in PAW-supplemented water exhibited elevated lysozyme activity and phagocytic indices, correlating with a 20-30% improvement in pathogen clearance. These immunomodulatory effects stem from hormetic dosing—low, non-cytotoxic RONS concentrations that trigger signaling cascades, including Nrf2 pathway activation, fostering cellular resilience.

The mechanistic underpinnings of PAW's immune enhancement involve targeted RONS interactions with fish immune cells. Hydroxyl radicals (•OH) and peroxynitrite (ONOO⁻), generated in situ, act as secondary messengers, stimulating leukocyte proliferation and cytokine release (e.g., IL-1β, TNF-α). In gill and skin epithelia—primary pathogen entry points—PAW induces mucin hypersecretion and tight junction reinforcement via MAPK/ERK signaling, forming a robust physical barrier. Complement system activation is also amplified, with alternative pathway components showing dose-dependent upregulation. Research on rainbow trout demonstrates that PAW bathing reduces bacterial adhesion by altering the surface glycocalyx, while intracellularly, H₂O₂ modulates redox-sensitive transcription factors, balancing pro- and anti-inflammatory responses. Critically, PAW avoids immunosuppression at optimal concentrations (ORP 400-600 mV, pH 5-6.5), preserving energy allocation for growth amid chronic stress. Synergy with dissolved oxygen from nanobubble integration further amplifies these benefits, as sustained hyperoxia supports ROS-dependent immune priming without hypoxia-induced lethargy.
Beyond immunity, PAW accelerates wound repair and tissue regeneration, which is vital for post-handling recovery in intensive farming. RONS orchestrate hemostasis, inflammation resolution, and proliferation phases: nitric oxide (NO•) from nitrate reduction promotes vasodilation and fibroblast migration, while H₂O₂ gradients guide macrophage polarization toward pro-regenerative M2 phenotypes. In zebrafish models, PAW immersion halved epithelial wound closure times via upregulated VEGF and collagen synthesis genes. Aquaculture trials with Atlantic salmon report 40% faster fin regeneration and reduced ulceration incidence in PAW-treated RAS, attributed to biofilm disruption on wounds that impedes healing. This regenerative capacity extends to stress mitigation, lowering cortisol levels and restoring hepatic glycogen stores disrupted by density or transport stressors.
Practical deployment in hatcheries and grow-out systems involves periodic PAW immersion (1:10 dilution, 30-60 min) or biofloc integration, yielding consistent health gains across species like carp, seabass, and shrimp. Field data show 15-25% mortality reductions during epizootics, alongside enhanced vaccine efficacy when PAW pre-treatments prime adaptive responses. However, dose optimization via real-time sensors is essential to avert pro-oxidant shifts in sensitive life stages.

Challenges persist, including species-specific responses and long-term bioaccumulation risks of nitrates. Future research should elucidate dose-response curves, gut microbiome interactions, and synergies with probiotics. Standardizing PAW formulations will unlock its full potential, transitioning aquaculture toward resilient, antibiotic-sparing paradigms.

3.3.3. Growth Promotion

Plasma activated water demonstrates significant potential in promoting growth and development across diverse aquatic organisms in aquaculture systems, extending its benefits beyond disinfection and immune enhancement to directly influence nutritional dynamics and metabolic efficiency. The unique chemical profile of plasma activated water, enriched with nitrates (NO₃⁻), nitrites (NO₂⁻), hydrogen peroxide (H₂O₂), and other reactive species, creates an optimized rearing environment that enhances primary productivity and feed utilization. Unlike conventional fertilizers, plasma activated water supplies nutrients in highly bioavailable forms without leaving chemical residues, thereby stimulating the proliferation of beneficial microalgae and phytoplankton that serve as natural feeds for filter feeding larvae, zooplankton, and herbivorous fish. This trophic enhancement increases food availability, reduces dependence on formulated feeds, and mitigates eutrophication risks associated with excess organic inputs. Experimental observations indicate that plasma activated water supplementation at suboptimal dilutions ranging from 1 to 5 percent volume per volume accelerates algal biomass accumulation by 20 to 50 percent within a few days. This response is attributed to enhanced nitrogen assimilation pathways that upregulate photosynthetic efficiency and chlorophyll synthesis in species such as Chlorella vulgaris and Isochrysis galbana. As a result, larval stages of shrimp such as Litopenaeus vannamei and finfish including seabass exhibit improved settlement success and early growth rates, effectively addressing nutritional limitations in intensive hatchery systems.

Direct physiological effects of plasma activated water on cultured species further highlight its growth promoting capacity. Exposure to low concentrations of reactive oxygen and nitrogen species induces hormetic responses that activate growth related signaling pathways, including insulin like growth factor 1 and the target of rapamycin pathway, thereby enhancing protein synthesis and somatic growth without causing oxidative stress. Studies on juvenile Oreochromis niloticus cultured in plasma activated water supplemented recirculating aquaculture systems report improvements of 15 to 25 percent in specific growth rate and overall weight gain during 60 day trials, accompanied by improved feed conversion ratios declining from 1.5 to 1.2. These gains are partly attributed to modulation of gut microbiota, where nitrite derived nitric oxide promotes the proliferation of beneficial Lactobacillus species, enhancing nutrient absorption and reducing metabolic losses associated with dysbiosis. Similar effects have been observed in Cyprinus carpio, where plasma activated water exposure enhances hepatic glycogenolysis and lipolysis during fasting stress, thereby sustaining anabolic metabolism. In Oncorhynchus mykiss, elevated expression of growth hormone receptors and myosin heavy chain genes has been documented, corresponding to a 10 to 20 percent increase in fillet yield accumulation. These responses are strongly dose dependent, emphasizing the importance of real time oxidation reduction potential monitoring to avoid cytotoxic thresholds. Additionally, the incorporation of microalgal biomass as a dietary supplement has been shown to further enhance growth performance and nutrient utilization in species such as Litopenaeus vannamei and Oreochromis niloticus (Vijayaram et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNyaXJlbmdhcmFqIiwiZmFtaWx5IjoiVmlqYXlhcmFtIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkthcnRoaWtleWFuIiwiZmFtaWx5IjoiTWFoZW5kcmFuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkVpbmFyIiwiZmFtaWx5IjoiUmluZ8O4In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhhcnkiLCJmYW1pbHkiOiJSYXphZmluZHJhbGFtYm8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU3VydWxpIiwiZmFtaWx5IjoiS2FubmFuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ill1biIsImZhbWlseSI6IlN1biJ9fV0sImlkcyI6eyJpc3NuIjpbIjE2NDItMzQwMiIsIjIzMDAtODczMyJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMjQ3OC9hb2FzLTIwMjQtMDExNSIsIm9wZW5hbGV4IjoiVzQ0MDQ3NTMwNDYiLCJvcGVuQWNjZXNzSWQiOiIxMjc3OTM2MjM3OTMxNTE3MDA2NCJ9LCJ0aXRsZSI6IkJpb2dlbmljIGRpZXRhcnkgcHJvbW90ZXJzIGluIGFxdWFjdWx0dXJlOiBuYXR1cmUtYmFzZWQgc29sdXRpb25zIGZvciBlbmhhbmNpbmcgZ3Jvd3RoLCBoZWFsdGgsIGFuZCBzdXN0YWluYWJpbGl0eSIsImFic3RyYWN0IjoiQWJzdHJhY3QgQXF1YWN1bHR1cmUsIGFzIGEgcmFwaWRseSBleHBhbmRpbmcgZ2xvYmFsIGluZHVzdHJ5LCBpcyBpbmNyZWFzaW5nbHkgY2hhbGxlbmdlZCBieSBib3RoIGluZmVjdGlvdXMgYW5kIG5vbi1pbmZlY3Rpb3VzIGRpc2Vhc2VzLCBwb3Npbmcgc2lnbmlmaWNhbnQgdGhyZWF0cyB0byBmaXNoIGhlYWx0aCBhbmQgcHJvZHVjdGlvbi4gVHJhZGl0aW9uYWwgaW50ZXJ2ZW50aW9ucywgc3VjaCBhcyBhbnRpYmlvdGljcyBhbmQgdmFjY2luZXMsIHRob3VnaCBlZmZlY3RpdmUsIHJhaXNlIGNvbmNlcm5zIHJlZ2FyZGluZyBlbnZpcm9ubWVudGFsIGltcGFjdCBhbmQgdGhlIHBvdGVudGlhbCBmb3IgcmVzaXN0YW5jZSBkZXZlbG9wbWVudC4gVGhpcyByZXZpZXcgZXhwbG9yZXMgdGhlIHByb21pc2luZyByb2xlIG9mIGRpZXRhcnkgYmlvZ2VuaWMgcHJvbW90ZXIgc3VwcGxlbWVudHMgYXMgYW4gaW5ub3ZhdGl2ZSwgc3VzdGFpbmFibGUgYWx0ZXJuYXRpdmUgZm9yIGFxdWFjdWx0dXJlLiBCeSBhbmFseXNpbmcgZW1lcmdpbmcgbGl0ZXJhdHVyZSwgd2UgaGlnaGxpZ2h0IHRoZSBzaWduaWZpY2FuY2Ugb2YgdGhlc2UgbmF0dXJlLWJhc2VkIHNvbHV0aW9ucywgd2hpY2ggb2ZmZXIgY29zdC1lZmZlY3RpdmUsIG5vbi10b3hpYywgYW5kIGVjby1mcmllbmRseSBvcHRpb25zIHRvIGVuaGFuY2UgYXF1YWN1bHR1cmUgcHJvZHVjdGl2aXR5LiBCaW9nZW5pYyBzdXBwbGVtZW50cywgZGVyaXZlZCBmcm9tIGhlcmJzLCBiZW5lZmljaWFsIG1pY3JvYmVzLCBpbnNlY3RzLCBudXRyaWVudHMsIGJpb2FjdGl2ZSBtb2xlY3VsZXMsIGFuZCBuYW5vcGFydGljbGVzLCBoYXZlIHNob3duIHBvdGVudGlhbCBpbiBpbXByb3ZpbmcgZmVlZCBxdWFsaXR5LCBwcm9tb3RpbmcgZ3Jvd3RoLCBib29zdGluZyBkaXNlYXNlIHJlc2lzdGFuY2UsIGVuaGFuY2luZyBpbW11bmUgc3lzdGVtIGZ1bmN0aW9uLCBhbmQgc3VwcG9ydGluZyBvdmVyYWxsIGhlYWx0aCBpbiBhcXVhdGljIHNwZWNpZXMuIFRoaXMgY29tcHJlaGVuc2l2ZSByZXZpZXcgZGVtb25zdHJhdGVzIHRoZSBlZmZpY2FjeSBvZiBiaW9nZW5pYy1tZWRpYXRlZCBzdXBwbGVtZW50cyBpbiBmb3N0ZXJpbmcgYXF1YWN1bHR1cmUgc3VzdGFpbmFiaWxpdHkgYnkgbWluaW1pemluZyB0aGUgbmVlZCBmb3IgdHJhZGl0aW9uYWwgY2hlbWljYWwgaW50ZXJ2ZW50aW9ucy4gRnVydGhlcm1vcmUsIHRoZSBhbmFseXNpcyBpZGVudGlmaWVzIGNyaXRpY2FsIHJlc2VhcmNoIGdhcHMsIHBhcnRpY3VsYXJseSBpbiBkZXRlcm1pbmluZyBvcHRpbWFsIHN1cHBsZW1lbnQgY29uY2VudHJhdGlvbnMgYW5kIHRoZWlyIHNwZWNpZXMtc3BlY2lmaWMgZWZmZWN0cy4gQWRkaXRpb25hbGx5LCBsYXJnZS1zY2FsZSBmaWVsZCB0cmlhbHMgYXJlIHJlY29tbWVuZGVkIHRvIHZhbGlkYXRlIGxhYm9yYXRvcnkgZmluZGluZ3MgYW5kIGFzc2VzcyB0aGVpciBwcmFjdGljYWwgYXBwbGljYXRpb25zIGluIHJlYWwtd29ybGQgc2V0dGluZ3MuIEJ5IHN5bnRoZXNpemluZyBjdXJyZW50IHJlc2VhcmNoLCB0aGlzIHJldmlldyBwcm92aWRlcyB2YWx1YWJsZSBpbnNpZ2h0cyBpbnRvIHRoZSBmdXR1cmUgdHJhamVjdG9yeSBvZiBhcXVhY3VsdHVyZSBwcmFjdGljZXMsIHBvc2l0aW9uaW5nIGJpb2dlbmljIGRpZXRhcnkgc3VwcGxlbWVudHMgYXMga2V5IGNvbnRyaWJ1dG9ycyB0byBzdXN0YWluYWJsZSBhbmQgZWZmaWNpZW50IGFxdWFjdWx0dXJlIGRldmVsb3BtZW50LiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4yNDc4L2FvYXMtMjAyNC0wMTE1IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3NjaWVuZG8uY29tL3BkZi8xMC4yNDc4L2FvYXMtMjAyNC0wMTE1IiwiaXNzdWVkRGF0ZSI6IjIwMjQtMTEtMjciLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA5IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJBbm5hbHMgb2YgQW5pbWFsIFNjaWVuY2UiLCJpbXBhY3RGYWN0b3IiOiIzLjM4In0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkRlIEdydXl0ZXIifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjIyMiwiY2l0ZWRCeUNvdW50IjoxfX0sInF1b3RlIjoiIG1pY3JvYWxnYWUgVGV0cmFzZWxtaXMgc3VlY2ljYSBMaXRvcGVuYWV1cyB2YW5uYW1laSBQYWNpZmljIHdoaXRlICBzaHJpbXAgNy41IGcva2cgSW1wcm92ZWQgZ3Jvd3RoICBhbmQgIG51dHJpZW50IHV0aWxpemF0aW9uIFNoYXJhd3kgZXQgIGFsLiAyMDIwIE1pY3JvYWxnYWUgQ2hsb3JlbGxhIHZ1bGdhcmlzIE1hY3JvYnJhY2hpdSBtIHJvc2VuYmVyZ2lpIEdpYW50IGZyZXNod2F0ZSByIHByYXduIDQlIGFuZCAxNSUgZm9yIDUwIGRheXMgIHRyaWFsIGJldHRlciBpbW11bmUgcmVzcG9uc2UgYW5kIHJlc2lzdGFuY2Ugb2YgTS4gcm9zZW5iZXJnaWkgcG9zdGxhcnZhZSBhZ2FpbnN0ICBBLiBoeWRyb3BoaWxhICBpbmZlY3Rpb24gTWFsaXdhdCBldCBhbC4gMjAxNyBNaWNyb2FsZ2FlIFNjZW5lZGVzbSB1cyBvYmxpcXV1cyBBbmFyaGljaGFzIG1pbm9yIFNwb3R0ZWQgd29sZmZpc2ggNCUgZm9yIDEyIHdlZWsgZmVlZGluZyAgdHJpYWxzIEluY3JlYXNlZCBtdXNjbGUgZ3Jvd3RoIEtudXRzZW5hIGV0IGFsLiAyMDE5IE1pY3JvYWxnYWUgR3JhY2lsYXJpYSAgYXJjdWF0YSBPcmVvY2hyb21pcyBuaWxvdGljdXMgTmlsZSB0aWxhcGlhIDIwJSBmb3IgYSAxMi13ZWVrIGZlZWRpbmcgdHJpYWwgSW1wcm92ZWQgZ3Jvd3RoIHBlcmZvcm1hbmNlIFlvdW5pcyBldCBhbC4gMjAxOCBNaWNyb2FsZ2FlIEFydGhyb3NwaXJhICBzcC4gU3BhcnVzIGF1cmF0YSBnaWx0aGVhZCAgc2VhYnJlYW0gNCUgZm9yIDEyOCBmZWVkaW5nIHRyaWFsICB0cmlhbHMgRW5oYW5jZWQgbXVzY2xlIHBpZ21lbnRhdGlvbiBhbmQgIGFudGlveGlkYW50ICBhY3Rpdml0eSBHYWxhZmF0IGV0IGFsLiAyMDIyIE1pY3JvYmVzIEJhY2lsbHVzICBwdW1pbHVzIE9yZW9jaHJvbWlzIG5pbG90aWN1cyBOaWxlIHRpbGFwaWEgMS44NSDDlyAxMDUgZm9yIDg0ICBmZWVkaW5nIHRyaWFscyBFbmhhbmNlZCB0aGUgZ3Jvd3RoIHBlcmZvcm1hbmNlLCAgaW50ZXN0aW5hbCAgaGlzdG9sb2dpY2FsICBtb3JwaG9tZXRyaWMsICBpbW11bmUgcmVzcG9uc2UsICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoyMiwiYmJveCI6eyJ0b3AiOjYzLjM1Mjc4MzIwMzEyNSwibGVmdCI6Ny43NDUwOTgxMTQwMTM2NzIsIndpZHRoIjo5LjE5OTM0NTU4ODY4NDA4MiwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MjIsImJib3giOnsidG9wIjo2MS4zMzc2Mjc0MTA4ODg2NywibGVmdCI6MTkuNjExMTEwNjg3MjU1ODYsIndpZHRoIjo5LjYxOTYwNzkyNTQxNTAzOSwiaGVpZ2h0Ijo0LjAzNjM2NTk4NTg3MDM2MX19LHsicGFnZU51bWJlciI6MjIsImJib3giOnsidG9wIjo2MS40OTIxNzYwNTU5MDgyLCJsZWZ0IjozMC42MzA3MjAxMzg1NDk4MDUsIndpZHRoIjoxMC4wNzI1NDYwMDUyNDkwMjMsImhlaWdodCI6My44ODE4MjA2Nzg3MTA5Mzc1fX0seyJwYWdlTnVtYmVyIjoyMiwiYmJveCI6eyJ0b3AiOjYxLjMzNzYyNzQxMDg4ODY3LCJsZWZ0Ijo0NC4wNDczODYxNjk0MzM1OTQsIndpZHRoIjo1Ljk0MTE3NDAzMDMwMzk1NSwiaGVpZ2h0Ijo2LjAzNjM2MTY5NDMzNTkzNzV9fSx7InBhZ2VOdW1iZXIiOjIyLCJiYm94Ijp7InRvcCI6NjEuMzM3NjI3NDEwODg4NjcsImxlZnQiOjUzLjkyOTczMzI3NjM2NzE5LCJ3aWR0aCI6MjcuODAxMzA3Njc4MjIyNjU2LCJoZWlnaHQiOjQuMDM2MzY1OTg1ODcwMzYxfX0seyJwYWdlTnVtYmVyIjoyMiwiYmJveCI6eyJ0b3AiOjY1LjM1Mjc4MzIwMzEyNSwibGVmdCI6NjYuNTk5NjcwNDEwMTU2MjUsIndpZHRoIjo4LjQ1MDk4MDE4NjQ2MjQwMiwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MjIsImJib3giOnsidG9wIjo2MS4zMzc2Mjc0MTA4ODg2NywibGVmdCI6ODMuMDE2MzQyMTYzMDg1OTQsIndpZHRoIjo5LjExMTc3MDYyOTg4MjgxMiwiaGVpZ2h0Ijo0LjAzNjM2NTk4NTg3MDM2MX19LHsicGFnZU51bWJlciI6MjIsImJib3giOnsidG9wIjo2NS4zNTI3ODMyMDMxMjUsImxlZnQiOjgzLjAxNjM0MjE2MzA4NTk0LCJ3aWR0aCI6NC40MTE3NjQ2MjE3MzQ2MTksImhlaWdodCI6Mi4wMjEyMTE2MjQxNDU1MDh9fSx7InBhZ2VOdW1iZXIiOjIyLCJiYm94Ijp7InRvcCI6NjguNjEwMzU5MTkxODk0NTMsImxlZnQiOjcuNzQ1MDk4MTE0MDEzNjcyLCJ3aWR0aCI6MjAuMzU2MjA4ODAxMjY5NTMsImhlaWdodCI6Mi4wMjEyMTE2MjQxNDU1MDh9fSx7InBhZ2VOdW1iZXIiOjIyLCJiYm94Ijp7InRvcCI6NzEuODgzMDc5NTI4ODA4NiwibGVmdCI6MTkuNjExMTEwNjg3MjU1ODYsIndpZHRoIjo2LjkyMTU2OTgyNDIxODc1LCJoZWlnaHQiOjIuMDIxMjExNjI0MTQ1NTA4fX0seyJwYWdlTnVtYmVyIjoyMiwiYmJveCI6eyJ0b3AiOjY4Ljc2NDkwNzgzNjkxNDA2LCJsZWZ0IjozMC42MzA3MjAxMzg1NDk4MDUsIndpZHRoIjoxMS40MzYyNzU0ODIxNzc3MzQsImhlaWdodCI6My44ODE4MTI4MTA4OTc4Mjd9fSx7InBhZ2VOdW1iZXIiOjIyLCJiYm94Ijp7InRvcCI6NjguNjEwMzU5MTkxODk0NTMsImxlZnQiOjQ0LjA0NzM4NjE2OTQzMzU5NCwid2lkdGgiOjcuNjA5ODA3NDkxMzAyNDksImhlaWdodCI6Ni4wMzYzNjE2OTQzMzU5Mzc1fX0seyJwYWdlTnVtYmVyIjoyMiwiYmJveCI6eyJ0b3AiOjY4LjYxMDM1OTE5MTg5NDUzLCJsZWZ0Ijo1My45Mjk3MzMyNzYzNjcxOSwid2lkdGgiOjExLjM2NjY2MTA3MTc3NzM0NCwiaGVpZ2h0Ijo2LjAzNjM2MTY5NDMzNTkzNzV9fSx7InBhZ2VOdW1iZXIiOjIyLCJiYm94Ijp7InRvcCI6NjguNjEwMzU5MTkxODk0NTMsImxlZnQiOjY2LjU5OTY3MDQxMDE1NjI1LCJ3aWR0aCI6MTUuMTM1Mjk2ODIxNTk0MjM4LCJoZWlnaHQiOjIuMDIxMjExNjI0MTQ1NTA4fX0seyJwYWdlTnVtYmVyIjoyMiwiYmJveCI6eyJ0b3AiOjcwLjYyNTUxMTE2OTQzMzYsImxlZnQiOjY2LjU5OTY3MDQxMDE1NjI1LCJ3aWR0aCI6MTUuMTM1MjkzMDA2ODk2OTczLCJoZWlnaHQiOjIuMDIxMjExNjI0MTQ1NTA4fX0seyJwYWdlTnVtYmVyIjoyMiwiYmJveCI6eyJ0b3AiOjcyLjYyNTUxMTE2OTQzMzYsImxlZnQiOjY2LjU5OTY3MDQxMDE1NjI1LCJ3aWR0aCI6MTUuMTM3MjU2NjIyMzE0NDUzLCJoZWlnaHQiOjIuMDIxMjExNjI0MTQ1NTA4fX0seyJwYWdlTnVtYmVyIjoyMiwiYmJveCI6eyJ0b3AiOjc0LjYyOTI5NTM0OTEyMTEsImxlZnQiOjY2LjU5OTY3MDQxMDE1NjI1LCJ3aWR0aCI6MTUuMTI5NDA3ODgyNjkwNDMsImhlaWdodCI6OC4wMzYzNTY5MjU5NjQzNTV9fSx7InBhZ2VOdW1iZXIiOjIyLCJiYm94Ijp7InRvcCI6NjguNjEwMzU5MTkxODk0NTMsImxlZnQiOjgzLjAxNjM0MjE2MzA4NTk0LCJ3aWR0aCI6OC42Mjc0NDkwMzU2NDQ1MzEsImhlaWdodCI6NC4wMzYzNTc4Nzk2Mzg2NzJ9fSx7InBhZ2VOdW1iZXIiOjIzLCJiYm94Ijp7InRvcCI6OC44NzY3NjgxMTIxODI2MTcsImxlZnQiOjcuNzQ1MDk4MTE0MDEzNjcyLCJ3aWR0aCI6MjEuMzM2NTk5MzQ5OTc1NTg2LCJoZWlnaHQiOjQuMDA0NTQ5MDI2NDg5MjU4fX0seyJwYWdlTnVtYmVyIjoyMywiYmJveCI6eyJ0b3AiOjkuMDMxMzEyOTQyNTA0ODgzLCJsZWZ0IjozMC42MzA3MjAxMzg1NDk4MDUsIndpZHRoIjo5LjUzMTM3MzAyMzk4NjgxNiwiaGVpZ2h0IjozLjg1MDAwMzI0MjQ5MjY3Nn19LHsicGFnZU51bWJlciI6MjMsImJib3giOnsidG9wIjo4Ljg3Njc2ODExMjE4MjYxNywibGVmdCI6NDQuMDQ3Mzg2MTY5NDMzNTk0LCJ3aWR0aCI6Ny4wMTk2MDYxMTM0MzM4MzgsImhlaWdodCI6NC4wMjEyMTU5MTU2Nzk5MzJ9fSx7InBhZ2VOdW1iZXIiOjIzLCJiYm94Ijp7InRvcCI6OC44NzY3NjgxMTIxODI2MTcsImxlZnQiOjUzLjkyOTczMzI3NjM2NzE5LCJ3aWR0aCI6MTEuMzY4NjI1NjQwODY5MTQsImhlaWdodCI6Ni4wMjEyMTE2MjQxNDU1MDh9fSx7InBhZ2VOdW1iZXIiOjIzLCJiYm94Ijp7InRvcCI6OC44NzY3NjgxMTIxODI2MTcsImxlZnQiOjY2LjU5OTY3MDQxMDE1NjI1LCJ3aWR0aCI6MTUuMTI5NDA3ODgyNjkwNDMsImhlaWdodCI6NC4wMjEyMTU5MTU2Nzk5MzJ9fSx7InBhZ2VOdW1iZXIiOjIzLCJiYm94Ijp7InRvcCI6OC44NzY3NjgxMTIxODI2MTcsImxlZnQiOjgzLjAxNjM0MjE2MzA4NTk0LCJ3aWR0aCI6OC42Mjc0NDkwMzU2NDQ1MzEsImhlaWdodCI6NC4wMjEyMTU5MTU2Nzk5MzJ9fSx7InBhZ2VOdW1iZXIiOjIzLCJiYm94Ijp7InRvcCI6MTYuMTQ5NDkyMjYzNzkzOTQ1LCJsZWZ0Ijo3Ljc0NTA5ODExNDAxMzY3Miwid2lkdGgiOjIwLjc0MDUyMDQ3NzI5NDkyMiwiaGVpZ2h0Ijo0LjAwNDU0OTUwMzMyNjQxNn19LHsicGFnZU51bWJlciI6MjMsImJib3giOnsidG9wIjoxNi4zMDQwMzcwOTQxMTYyMSwibGVmdCI6MzAuNjMwNzIwMTM4NTQ5ODA1LCJ3aWR0aCI6MTAuODE5NjA4Njg4MzU0NDkyLCJoZWlnaHQiOjMuODUwMDA0NDM0NTg1NTcxM319LHsicGFnZU51bWJlciI6MjMsImJib3giOnsidG9wIjoxNi4xNDk0OTIyNjM3OTM5NDUsImxlZnQiOjQ0LjA0NzM4NjE2OTQzMzU5NCwid2lkdGgiOjUuMzkyMTU3MDc3Nzg5MzA3LCJoZWlnaHQiOjQuMDIxMjE0OTYyMDA1NjE1fX0seyJwYWdlTnVtYmVyIjoyMywiYmJveCI6eyJ0b3AiOjE2LjE0OTQ5MjI2Mzc5Mzk0NSwibGVmdCI6NTMuOTI5NzMzMjc2MzY3MTksIndpZHRoIjoxMS4zNjQ3MDYwMzk0Mjg3MTEsImhlaWdodCI6Ni4wMzYzNjkzMjM3MzA0Njl9fSx7InBhZ2VOdW1iZXIiOjIzLCJiYm94Ijp7InRvcCI6MTYuMTQ5NDkyMjYzNzkzOTQ1LCJsZWZ0Ijo2Ni41OTk2NzA0MTAxNTYyNSwid2lkdGgiOjE1LjEyOTQxNDU1ODQxMDY0NSwiaGVpZ2h0Ijo0LjAyMTIxNDk2MjAwNTYxNX19LHsicGFnZU51bWJlciI6MjMsImJib3giOnsidG9wIjoxNi4xNDk0OTIyNjM3OTM5NDUsImxlZnQiOjgzLjAxNjM0MjE2MzA4NTk0LCJ3aWR0aCI6OC42MjE1NzUzNTU1Mjk3ODUsImhlaWdodCI6NC4wMjEyMTQ5NjIwMDU2MTV9fSx7InBhZ2VOdW1iZXIiOjIzLCJiYm94Ijp7InRvcCI6MjMuNDIyMjI0MDQ0Nzk5ODA1LCJsZWZ0Ijo3Ljc0NTA5ODExNDAxMzY3Miwid2lkdGgiOjIxLjYwNzE4OTE3ODQ2Njc5NywiaGVpZ2h0Ijo0LjAwNDU0MjM1MDc2OTA0M319LHsicGFnZU51bWJlciI6MjMsImJib3giOnsidG9wIjoyMy40MjIyMjQwNDQ3OTk4MDUsImxlZnQiOjMwLjYzMDcyMDEzODU0OTgwNSwid2lkdGgiOjIxLjMxODYyNDQ5NjQ1OTk2LCJoZWlnaHQiOjQuMDIxMjA3MzMyNjExMDg0fX0seyJwYWdlTnVtYmVyIjoyMywiYmJveCI6eyJ0b3AiOjIzLjQyMjIyNDA0NDc5OTgwNSwibGVmdCI6NTMuOTI5NzMzMjc2MzY3MTksIndpZHRoIjoxMS4zNjY2NjU4NDAxNDg5MjYsImhlaWdodCI6Ni4wMzYzNjE2OTQzMzU5Mzc1fX0seyJwYWdlTnVtYmVyIjoyMywiYmJveCI6eyJ0b3AiOjIzLjQyMjIyNDA0NDc5OTgwNSwibGVmdCI6NjYuNTk5NjcwNDEwMTU2MjUsIndpZHRoIjoxNS4xMzMzMzMyMDYxNzY3NTgsImhlaWdodCI6OC4wMzYzNjQ1NTUzNTg4ODd9fSx7InBhZ2VOdW1iZXIiOjIzLCJiYm94Ijp7InRvcCI6MjMuNDIyMjI0MDQ0Nzk5ODA1LCJsZWZ0Ijo4My4wMTYzNDIxNjMwODU5NCwid2lkdGgiOjguNjIzNTM5OTI0NjIxNTgyLCJoZWlnaHQiOjQuMDIxMjA3MzMyNjExMDg0fX0seyJwYWdlTnVtYmVyIjoyMywiYmJveCI6eyJ0b3AiOjMyLjY5ODczODA5ODE0NDUzLCJsZWZ0Ijo3Ljc0NTA5ODExNDAxMzY3Miwid2lkdGgiOjE4Ljc3OTczNTU2NTE4NTU0NywiaGVpZ2h0Ijo0LjAxOTY5NjcxMjQ5Mzg5NjV9fSx7InBhZ2VOdW1iZXIiOjIzLCJiYm94Ijp7InRvcCI6MzIuODUzMjgyOTI4NDY2OCwibGVmdCI6MzAuNjMwNzIwMTM4NTQ5ODA1LCJ3aWR0aCI6MTAuODE5NjA4Njg4MzU0NDkyLCJoZWlnaHQiOjMuODY1MTUxNDA1MzM0NDcyN319LHsicGFnZU51bWJlciI6MjMsImJib3giOnsidG9wIjozMi42OTg3MzgwOTgxNDQ1MywibGVmdCI6NDQuMDQ3Mzg2MTY5NDMzNTk0LCJ3aWR0aCI6NS4zOTIxNTcwNzc3ODkzMDcsImhlaWdodCI6NC4wMzYzNjU5ODU4NzAzNjF9fSx7InBhZ2VOdW1iZXIiOjIzLCJiYm94Ijp7InRvcCI6MzIuNjQ5NDc4OTEyMzUzNTE2LCJsZWZ0Ijo1My45Mjk3MzMyNzYzNjcxOSwid2lkdGgiOjExLjM3NTgyMTExMzU4NjQyNiwiaGVpZ2h0Ijo0LjA4NTYyNTY0ODQ5ODUzNX19LHsicGFnZU51bWJlciI6MjMsImJib3giOnsidG9wIjozNi43MTM4OTM4OTAzODA4NiwibGVmdCI6NTMuOTI5NzMzMjc2MzY3MTksIndpZHRoIjo0LjQxNTAzNjIwMTQ3NzA1MSwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MjMsImJib3giOnsidG9wIjozMi42OTg3MzgwOTgxNDQ1MywibGVmdCI6NjYuNTk5NjcwNDEwMTU2MjUsIndpZHRoIjoxNS4xMzMzMzMyMDYxNzY3NTgsImhlaWdodCI6Mi4wMjEyMTE2MjQxNDU1MDh9fSx7InBhZ2VOdW1iZXIiOjIzLCJiYm94Ijp7InRvcCI6MzQuNzEzODkwMDc1NjgzNTk0LCJsZWZ0Ijo2Ni41OTk2NzA0MTAxNTYyNSwid2lkdGgiOjYuMDM3MjU0ODEwMzMzMjUyLCJoZWlnaHQiOjIuMDIxMjExNjI0MTQ1NTA4fX0seyJwYWdlTnVtYmVyIjoyMywiYmJveCI6eyJ0b3AiOjM2LjcxMzg5Mzg5MDM4MDg2LCJsZWZ0Ijo2Ni41OTk2NzA0MTAxNTYyNSwid2lkdGgiOjEwLjg2MDc4NzM5MTY2MjU5OCwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MjMsImJib3giOnsidG9wIjozOC43MTM4OTAwNzU2ODM1OTQsImxlZnQiOjY2LjU5OTY3MDQxMDE1NjI1LCJ3aWR0aCI6Ny42Nzg0MzE5ODc3NjI0NTEsImhlaWdodCI6Mi4wMjEyMTE2MjQxNDU1MDh9fSx7InBhZ2VOdW1iZXIiOjIzLCJiYm94Ijp7InRvcCI6NDAuNzEzODkwMDc1NjgzNTk0LCJsZWZ0Ijo2Ni41OTk2NzA0MTAxNTYyNSwid2lkdGgiOjkuNjIxNTY4Njc5ODA5NTcsImhlaWdodCI6Mi4wMjEyMTE2MjQxNDU1MDh9fSx7InBhZ2VOdW1iZXIiOjIzLCJiYm94Ijp7InRvcCI6NDIuNzI5MDQyMDUzMjIyNjU2LCJsZWZ0Ijo2Ni41OTk2NzA0MTAxNTYyNSwid2lkdGgiOjEyLjA3ODQyODI2ODQzMjYxNywiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MjMsImJib3giOnsidG9wIjo0NC43MjkwNDU4Njc5MTk5MiwibGVmdCI6NjYuNTk5NjcwNDEwMTU2MjUsIndpZHRoIjoxNS4xMzcyNTY2MjIzMTQ0NTMsImhlaWdodCI6Mi4wMjEyMTE2MjQxNDU1MDh9fV19LCJjaHVua0lkIjoiZmUxZTJmODEtZDk5Yy00OWVjLWE1Y2MtN2U2M2MwN2YwMzEyIiwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiMjMifX1d.

Synergistic integration of PAW with nanobubble oxygenation amplifies growth outcomes by sustaining hyperoxic conditions that fuel aerobic metabolism (Tekile et al., 2016)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFuZGluZXQgS2ViZWRlIiwiZmFtaWx5IjoiVGVraWxlIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikls4oCQSG8iLCJmYW1pbHkiOiJLaW0ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSmFpeWVvcCIsImZhbWlseSI6IkxlZSJ9fV0sImlkcyI6eyJpc3NuIjpbIjEyMjYtMTAyNSIsIjIwMDUtOTY4WCJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuNDQ5MS9lZXIuMjAxNi4wMjgiLCJtYWciOiIyNTIzNjY5NjU3Iiwib3BlbmFsZXgiOiJXMjUyMzY2OTY1NyJ9LCJ0aXRsZSI6IkV4dGVudCBhbmQgcGVyc2lzdGVuY2Ugb2YgZGlzc29sdmVkIG94eWdlbiBlbmhhbmNlbWVudCB1c2luZyBuYW5vYnViYmxlcyIsImFic3RyYWN0IjoiSW4gdGhpcyBzdHVkeSwgY2hhbmdlIGluIHdhdGVyLWRpc3NvbHZlZCBveHlnZW4gKERPKSB3YXMgYW5hbHl6ZWQgdW5kZXIgdmFyaW91cyBzeW50aGV0aWMgd2F0ZXIgcXVhbGl0aWVzIGFuZCBuYW5vYnViYmxlcyAoTkJzKSBhcHBsaWNhdGlvbiBjb25kaXRpb25zLCBzdWNoIGFzIGdhcyB0eXBlLCBpbml0aWFsIERPIGFzIHdlbGwgYXMgd2F0ZXIgZGlzc29sdmVkLCBzdXNwZW5kZWQgYW5kIG9yZ2FuaWMgbWF0dGVycyBjb250ZW50cy4gV2hlbiBveHlnZW4sIHJhdGhlciB0aGFuIGFpciwgd2FzIGludHJvZHVjZWQgaW50byBuaXRyb2dlbi1kZXNvcmJlZCB1bHRyYS1wdXJlIHdhdGVyLCB0aGUgc3RhZ25hdGlvbiB0aW1lIHdhcyBzaWduaWZpY2FudGx5IGluY3JlYXNlZC4gSXQgdG9vayB0ZW4gZGF5cyBmb3IgRE8gY29uY2VudHJhdGlvbiB0byBkcm9wIGJhY2sgdG8gc2F0dXJhdGlvbi4gVGhlIGhpZ2hlciB0aGUgaW5pdGlhbCBETyBjb25jZW50cmF0aW9uLCB0aGUgbG9uZ2VyIHBhcnRpY2xlcyB3ZXJlIG9ic2VydmVkIGFib3ZlIHNhdHVyYXRpb24gZHVlIHRvIHBhcnRpY2xlIHN0YWJpbGl0eSBpbXByb3ZlbWVudC4gVGhlIG94eWdlbiBtYXNzIHRyYW5zZmVyIHJhdGUgb2YgMC4wNDgyIG1nL0wvbWluIHdhcyBmb3VuZCB0byByZWFjaCBhIG1heGltdW0gYXQgYW4gZWxlY3Ryb2x5dGljIGNvbmNlbnRyYXRpb24gb2YgMC43NSBnL0wsIGJleW9uZCB3aGljaCB0aGUgdHJhbnNmZXIgcmF0ZSBkZWNyZWFzZWQgZHVlIHRvIGFkc29ycHRpb24gb2YgbmVnYXRpdmUgaW9ucyBvZiB0aGUgZWxlY3Ryb2x5dGUgYXQgdGhlIGludGVyZmFjZS4gSGlnaCBsZXZlbHMgb2YgdHVyYmlkaXR5IGNhdXNlZCBieSBzdXNwZW5kZWQgc29saWRzIGhhdmUgYmVjb21lIGEgYmFycmllciB0byBkaXNzb2x1dGlvbiBvZiBOQnMgb3h5Z2VuIGludG8gdGhlIHdhdGVyIHNvbHV0aW9uLCBhbmQgdGh1cyBhZmZlY3RlZCB0aGUgdHJhbnNmZXIgcGVyZm9ybWFuY2UuIE9uIHRoZSBvdGhlciBoYW5kLCBieSBhcHBseWluZyBOQnMgZm9yIGp1c3QgYW4gaG91ciwgdXAgdG8gNy4yJSBkZWdyYWRhdGlvbiBvZiBnbHVjb3NlIGFzIHJlcHJlc2VudGF0aXZlIG9yZ2FuaWMgbWF0dGVyIHdhcyBhY2hpZXZlZC4gVGh1cywgTkJzIHRlY2hub2xvZ3kgd291bGQgbWFpbnRhaW4gYSBoaWdoIERPIGV4dGVudCBmb3IgYW4gZXh0ZW5kZWQgZHVyYXRpb24sIGFuZCB0aHVzIGNhbiBpbXByb3ZlIHdhdGVyIHF1YWxpdHkgcHJvdmlkZWQgdGhhdCB3YXRlciBjaGVtaXN0cnkgaXMgY2xvc2VseSBtb25pdG9yZWQgZHVyaW5nIGl0cyBhcHBsaWNhdGlvbi4gS2V5d29yZHM6IEFwcGxpY2F0aW9uIGNvbmRpdGlvbnMsIERpc3NvbHZlZCBveHlnZW4sIE5hbm9idWJibGVzLCBXYXRlciBxdWFsaXR5IiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjQ0OTEvZWVyLjIwMTYuMDI4IiwicmVzb3VyY2VVcmwiOiJodHRwOi8vZWVlci5vcmcvdXBsb2FkL2Vlci0yMS00LTQyNy5wZGYiLCJpc3N1ZWREYXRlIjoiMjAxNi0wOS0xMiIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkVudmlyb25tZW50YWwgRW5naW5lZXJpbmcgUmVzZWFyY2giLCJpc3N1ZSI6NCwidm9sdW1lIjoyMSwiaW1wYWN0RmFjdG9yIjoiNC4wNSJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjQyNywibGFzdFBhZ2UiOjQzNX19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJLb3JlYW4gU29jaWV0eSBvZiBFbnZpcm9ubWVudGFsIEVuZ2luZWVyaW5nIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjoyNCwiY2l0ZWRCeUNvdW50IjozM319LCJxdW90ZSI6IlRoaXMgdmVyaWZpZXMgdGhlIHBvc3NpYmlsaXR5IG9mIHRoZSBleGlzdGVuY2UgIG9mIE5Ccywgc2luY2UgYXQgYSBoaWdoZXIgRE8gY29uY2VudHJhdGlvbiwgdGhlIG94eWdlbiBkaWZmdXNpb24gIHdvdWxkIHRha2UgbW9yZSB0aW1lLiBUaGUgaGlnaCBnYXMgY29uY2VudHJhdGlvbiBpbiB0aGUgbGlxdWlkICBleHBsYWlucyB0aGUgZXh0ZW5zaW9uIG9mIE5CIHN0YWJpbGl0eS4gIFRoZSBtYWduaXR1ZGVzIG9mIHpldGEgcG90ZW50aWFsIG9mIG94eWdlbiBuYW5vLWJ1YmJsZWQgd2F0ZXIgIHdlcmUgcG9zc2libHkgaGlnaGVyIHRoYW4gdGhvc2Ugb2YgYWlyIG5hbm8tYnViYmxlZCB3YXRlci4gQXQgIGEgaGlnaCBtYWduaXR1ZGUgemV0YSBwb3RlbnRpYWwgdmFsdWUsIHRoZSBlbGVjdHJpY2FsbHkgY2hhcmdlZCAgcGFydGljbGVzIHRlbmQgdG8gcmVwZWwgZWFjaCBvdGhlciwgYXZvaWRpbmcgYWdncmVnYXRpb24uIEZvciBidWJibGUgIGRpc3BlcnNpb24sIHRoZSBoaWdoIHpldGEgcG90ZW50aWFsIGNvdWxkIGNyZWF0ZSByZXB1bHNpb24gZm9yY2VzICB0aGF0IHdvdWxkIGF2b2lkIHRoZSBjb2FsZXNjZW5jZSBvZiBidWJibGVzLCB0aHVzIGNvbnRyaWJ1dGluZyAgdG8gdGhlIHN0YWJpbGl6YXRpb24gb2YgdGhlIGJ1YmJsZXMuIFRoZXJlZm9yZSwgdGhlIG94eWdlbiBOQnMgIGNvdWxkIGJlIHN0YWJpbGl6ZWQgaW4gd2F0ZXIgYnkgdGhlIGVsZWN0cmljYWxseSBjaGFyZ2VkIHN1cmZhY2VzLCAgd2hpbGUgdGhlIG5hbm8tYnViYmxlZCBhaXIgd291bGQgYmUgYmVsb3cgdGhlIGxpbWl0IG9mIHRoZSBzdGFiaWxpemEtdGlvbiBieSByZXB1bHNpb24gZm9yY2VzLiBGdXJ0aGVybW9yZSwgdGhlIGJpZyBkaWZmZXJlbmNlIGJldHdlZW4gdGhlIHBlcmlvZHMgaW4gd2hpY2ggIGJ1YmJsZXMgY291bGQgYmUgb2JzZXJ2ZWQgYXMgc3RhYmxlIGluIHRoZSBveHlnZW4gbmFuby1idWJibGVkICB3YXRlciAoMTAtMTUgZCkgYW5kIGluIHRoYXQgb2YgYWlyIG5hbm8tYnViYmxlZCAoMy01IGQpIGlzIGxpa2VseSBGaWcuIDYuIEdyYXBocyBzaG93aW5nIHN0YWduYXRpb24gdGltZSBmb3IgZGlmZmVyZW50IGluaXRpYWwgZGlzc29sdmVkIG94eWdlbiAoRE8pLiByZWxhdGVkIHRvIHRoZSBsb3dlciBkaXNzb2x1dGlvbiBvZiB0aGUgYWlyIGluIHdhdGVyIGluIHJlbGF0aW9uIHRvIHRoZSBkaXNzb2x1dGlvbiBvZiBveHlnZW4uICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6NjYuMTc4ODAyNDkwMjM0MzgsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6NDAuNzE0ODg5NTI2MzY3MTksImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo2Ny41NzY3NzQ1OTcxNjc5NywibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42ODgyNzA1Njg4NDc2NTYsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo2OC45NjcyMzE3NTA0ODgyOCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NTcxODA3ODYxMzI4MSwiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjcwLjM1NzY4ODkwMzgwODYsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6MjUuMzQxMjU1MTg3OTg4MjgsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3MS43NTU2NTMzODEzNDc2NiwibGVmdCI6OS41NDk2NDE2MDkxOTE4OTUsIndpZHRoIjozOS4wMjQ3ODQwODgxMzQ3NjYsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3My4xNDYxMDI5MDUyNzM0NCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NTQwMjk4NDYxOTE0MDYsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3NC41MzY1NjAwNTg1OTM3NSwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NTM0NjUyNzA5OTYwOTQsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3NS45MzQ1MzIxNjU1MjczNCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NzMyMzY4NDY5MjM4MywiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjc3LjMyNDk4OTMxODg0NzY2LCJsZWZ0Ijo3Ljg2NTU0NTI3MjgyNzE0OCwid2lkdGgiOjQwLjY0OTM1MzAyNzM0Mzc1LCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6NzguNzE1NDQ2NDcyMTY3OTcsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6NDAuNjU0NjI0OTM4OTY0ODQ0LCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODAuMTEzNDEwOTQ5NzA3MDMsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6NDAuNjUxMDUwNTY3NjI2OTUsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo4MS41MDM4NjA0NzM2MzI4MSwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NTQzNDI2NTEzNjcxOSwiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjgyLjg5NDMxNzYyNjk1MzEyLCJsZWZ0Ijo3Ljg2NTU0NTI3MjgyNzE0OCwid2lkdGgiOjQwLjMyMTgyMzEyMDExNzE5LCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODQuMjkyMjg5NzMzODg2NzIsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6MTUuODcwMjk1NTI0NTk3MTY4LCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODUuNjgyNzQ2ODg3MjA3MDMsImxlZnQiOjkuNTQ5NjQxNjA5MTkxODk1LCJ3aWR0aCI6MzguOTg5NzA0MTMyMDgwMDgsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo4Ny4wNzMyMDQwNDA1MjczNCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC43MTg0OTQ0MTUyODMyLCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODguNDcxMTY4NTE4MDY2NCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42ODc0MTk4OTEzNTc0MiwiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjM0LjUxMzI3MTMzMTc4NzExLCJsZWZ0Ijo1MS45MzI3Nzc0MDQ3ODUxNTYsIndpZHRoIjo0MC42NDcwNTI3NjQ4OTI1OCwiaGVpZ2h0IjoyLjcxODY2NjU1MzQ5NzMxNDV9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjozOC43MjUxNTg2OTE0MDYyNSwibGVmdCI6NTEuOTMyNzc3NDA0Nzg1MTU2LCJ3aWR0aCI6NDAuNjQzMTM1MDcwODAwNzgsImhlaWdodCI6MS4yNTMyODg1MDc0NjE1NDc5fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6NDAuMTE1NjEyMDMwMDI5MywibGVmdCI6NTEuOTMyNzc3NDA0Nzg1MTU2LCJ3aWR0aCI6NDAuNjY0NTg1MTEzNTI1MzksImhlaWdodCI6MS4yNTMyODg1MDc0NjE1NDc5fX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNDMxIn19XQ%3D%3D. Nanobubbles' prolonged dissolved oxygen elevation, stabilized by high zeta potential repulsion, complements PAW's nitrate-driven primary production, creating a nutrient-oxygen nexus that boosts overall system productivity (Tekile et al., 2016)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFuZGluZXQgS2ViZWRlIiwiZmFtaWx5IjoiVGVraWxlIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikls4oCQSG8iLCJmYW1pbHkiOiJLaW0ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSmFpeWVvcCIsImZhbWlseSI6IkxlZSJ9fV0sImlkcyI6eyJpc3NuIjpbIjEyMjYtMTAyNSIsIjIwMDUtOTY4WCJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuNDQ5MS9lZXIuMjAxNi4wMjgiLCJtYWciOiIyNTIzNjY5NjU3Iiwib3BlbmFsZXgiOiJXMjUyMzY2OTY1NyJ9LCJ0aXRsZSI6IkV4dGVudCBhbmQgcGVyc2lzdGVuY2Ugb2YgZGlzc29sdmVkIG94eWdlbiBlbmhhbmNlbWVudCB1c2luZyBuYW5vYnViYmxlcyIsImFic3RyYWN0IjoiSW4gdGhpcyBzdHVkeSwgY2hhbmdlIGluIHdhdGVyLWRpc3NvbHZlZCBveHlnZW4gKERPKSB3YXMgYW5hbHl6ZWQgdW5kZXIgdmFyaW91cyBzeW50aGV0aWMgd2F0ZXIgcXVhbGl0aWVzIGFuZCBuYW5vYnViYmxlcyAoTkJzKSBhcHBsaWNhdGlvbiBjb25kaXRpb25zLCBzdWNoIGFzIGdhcyB0eXBlLCBpbml0aWFsIERPIGFzIHdlbGwgYXMgd2F0ZXIgZGlzc29sdmVkLCBzdXNwZW5kZWQgYW5kIG9yZ2FuaWMgbWF0dGVycyBjb250ZW50cy4gV2hlbiBveHlnZW4sIHJhdGhlciB0aGFuIGFpciwgd2FzIGludHJvZHVjZWQgaW50byBuaXRyb2dlbi1kZXNvcmJlZCB1bHRyYS1wdXJlIHdhdGVyLCB0aGUgc3RhZ25hdGlvbiB0aW1lIHdhcyBzaWduaWZpY2FudGx5IGluY3JlYXNlZC4gSXQgdG9vayB0ZW4gZGF5cyBmb3IgRE8gY29uY2VudHJhdGlvbiB0byBkcm9wIGJhY2sgdG8gc2F0dXJhdGlvbi4gVGhlIGhpZ2hlciB0aGUgaW5pdGlhbCBETyBjb25jZW50cmF0aW9uLCB0aGUgbG9uZ2VyIHBhcnRpY2xlcyB3ZXJlIG9ic2VydmVkIGFib3ZlIHNhdHVyYXRpb24gZHVlIHRvIHBhcnRpY2xlIHN0YWJpbGl0eSBpbXByb3ZlbWVudC4gVGhlIG94eWdlbiBtYXNzIHRyYW5zZmVyIHJhdGUgb2YgMC4wNDgyIG1nL0wvbWluIHdhcyBmb3VuZCB0byByZWFjaCBhIG1heGltdW0gYXQgYW4gZWxlY3Ryb2x5dGljIGNvbmNlbnRyYXRpb24gb2YgMC43NSBnL0wsIGJleW9uZCB3aGljaCB0aGUgdHJhbnNmZXIgcmF0ZSBkZWNyZWFzZWQgZHVlIHRvIGFkc29ycHRpb24gb2YgbmVnYXRpdmUgaW9ucyBvZiB0aGUgZWxlY3Ryb2x5dGUgYXQgdGhlIGludGVyZmFjZS4gSGlnaCBsZXZlbHMgb2YgdHVyYmlkaXR5IGNhdXNlZCBieSBzdXNwZW5kZWQgc29saWRzIGhhdmUgYmVjb21lIGEgYmFycmllciB0byBkaXNzb2x1dGlvbiBvZiBOQnMgb3h5Z2VuIGludG8gdGhlIHdhdGVyIHNvbHV0aW9uLCBhbmQgdGh1cyBhZmZlY3RlZCB0aGUgdHJhbnNmZXIgcGVyZm9ybWFuY2UuIE9uIHRoZSBvdGhlciBoYW5kLCBieSBhcHBseWluZyBOQnMgZm9yIGp1c3QgYW4gaG91ciwgdXAgdG8gNy4yJSBkZWdyYWRhdGlvbiBvZiBnbHVjb3NlIGFzIHJlcHJlc2VudGF0aXZlIG9yZ2FuaWMgbWF0dGVyIHdhcyBhY2hpZXZlZC4gVGh1cywgTkJzIHRlY2hub2xvZ3kgd291bGQgbWFpbnRhaW4gYSBoaWdoIERPIGV4dGVudCBmb3IgYW4gZXh0ZW5kZWQgZHVyYXRpb24sIGFuZCB0aHVzIGNhbiBpbXByb3ZlIHdhdGVyIHF1YWxpdHkgcHJvdmlkZWQgdGhhdCB3YXRlciBjaGVtaXN0cnkgaXMgY2xvc2VseSBtb25pdG9yZWQgZHVyaW5nIGl0cyBhcHBsaWNhdGlvbi4gS2V5d29yZHM6IEFwcGxpY2F0aW9uIGNvbmRpdGlvbnMsIERpc3NvbHZlZCBveHlnZW4sIE5hbm9idWJibGVzLCBXYXRlciBxdWFsaXR5IiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjQ0OTEvZWVyLjIwMTYuMDI4IiwicmVzb3VyY2VVcmwiOiJodHRwOi8vZWVlci5vcmcvdXBsb2FkL2Vlci0yMS00LTQyNy5wZGYiLCJpc3N1ZWREYXRlIjoiMjAxNi0wOS0xMiIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkVudmlyb25tZW50YWwgRW5naW5lZXJpbmcgUmVzZWFyY2giLCJpc3N1ZSI6NCwidm9sdW1lIjoyMSwiaW1wYWN0RmFjdG9yIjoiNC4wNSJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjQyNywibGFzdFBhZ2UiOjQzNX19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJLb3JlYW4gU29jaWV0eSBvZiBFbnZpcm9ubWVudGFsIEVuZ2luZWVyaW5nIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjoyNCwiY2l0ZWRCeUNvdW50IjozM319LCJxdW90ZSI6IlRoaXMgdmVyaWZpZXMgdGhlIHBvc3NpYmlsaXR5IG9mIHRoZSBleGlzdGVuY2UgIG9mIE5Ccywgc2luY2UgYXQgYSBoaWdoZXIgRE8gY29uY2VudHJhdGlvbiwgdGhlIG94eWdlbiBkaWZmdXNpb24gIHdvdWxkIHRha2UgbW9yZSB0aW1lLiBUaGUgaGlnaCBnYXMgY29uY2VudHJhdGlvbiBpbiB0aGUgbGlxdWlkICBleHBsYWlucyB0aGUgZXh0ZW5zaW9uIG9mIE5CIHN0YWJpbGl0eS4gIFRoZSBtYWduaXR1ZGVzIG9mIHpldGEgcG90ZW50aWFsIG9mIG94eWdlbiBuYW5vLWJ1YmJsZWQgd2F0ZXIgIHdlcmUgcG9zc2libHkgaGlnaGVyIHRoYW4gdGhvc2Ugb2YgYWlyIG5hbm8tYnViYmxlZCB3YXRlci4gQXQgIGEgaGlnaCBtYWduaXR1ZGUgemV0YSBwb3RlbnRpYWwgdmFsdWUsIHRoZSBlbGVjdHJpY2FsbHkgY2hhcmdlZCAgcGFydGljbGVzIHRlbmQgdG8gcmVwZWwgZWFjaCBvdGhlciwgYXZvaWRpbmcgYWdncmVnYXRpb24uIEZvciBidWJibGUgIGRpc3BlcnNpb24sIHRoZSBoaWdoIHpldGEgcG90ZW50aWFsIGNvdWxkIGNyZWF0ZSByZXB1bHNpb24gZm9yY2VzICB0aGF0IHdvdWxkIGF2b2lkIHRoZSBjb2FsZXNjZW5jZSBvZiBidWJibGVzLCB0aHVzIGNvbnRyaWJ1dGluZyAgdG8gdGhlIHN0YWJpbGl6YXRpb24gb2YgdGhlIGJ1YmJsZXMuIFRoZXJlZm9yZSwgdGhlIG94eWdlbiBOQnMgIGNvdWxkIGJlIHN0YWJpbGl6ZWQgaW4gd2F0ZXIgYnkgdGhlIGVsZWN0cmljYWxseSBjaGFyZ2VkIHN1cmZhY2VzLCAgd2hpbGUgdGhlIG5hbm8tYnViYmxlZCBhaXIgd291bGQgYmUgYmVsb3cgdGhlIGxpbWl0IG9mIHRoZSBzdGFiaWxpemEtdGlvbiBieSByZXB1bHNpb24gZm9yY2VzLiBGdXJ0aGVybW9yZSwgdGhlIGJpZyBkaWZmZXJlbmNlIGJldHdlZW4gdGhlIHBlcmlvZHMgaW4gd2hpY2ggIGJ1YmJsZXMgY291bGQgYmUgb2JzZXJ2ZWQgYXMgc3RhYmxlIGluIHRoZSBveHlnZW4gbmFuby1idWJibGVkICB3YXRlciAoMTAtMTUgZCkgYW5kIGluIHRoYXQgb2YgYWlyIG5hbm8tYnViYmxlZCAoMy01IGQpIGlzIGxpa2VseSBGaWcuIDYuIEdyYXBocyBzaG93aW5nIHN0YWduYXRpb24gdGltZSBmb3IgZGlmZmVyZW50IGluaXRpYWwgZGlzc29sdmVkIG94eWdlbiAoRE8pLiByZWxhdGVkIHRvIHRoZSBsb3dlciBkaXNzb2x1dGlvbiBvZiB0aGUgYWlyIGluIHdhdGVyIGluIHJlbGF0aW9uIHRvIHRoZSBkaXNzb2x1dGlvbiBvZiBveHlnZW4uICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6NjYuMTc4ODAyNDkwMjM0MzgsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6NDAuNzE0ODg5NTI2MzY3MTksImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo2Ny41NzY3NzQ1OTcxNjc5NywibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42ODgyNzA1Njg4NDc2NTYsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo2OC45NjcyMzE3NTA0ODgyOCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NTcxODA3ODYxMzI4MSwiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjcwLjM1NzY4ODkwMzgwODYsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6MjUuMzQxMjU1MTg3OTg4MjgsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3MS43NTU2NTMzODEzNDc2NiwibGVmdCI6OS41NDk2NDE2MDkxOTE4OTUsIndpZHRoIjozOS4wMjQ3ODQwODgxMzQ3NjYsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3My4xNDYxMDI5MDUyNzM0NCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NTQwMjk4NDYxOTE0MDYsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3NC41MzY1NjAwNTg1OTM3NSwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NTM0NjUyNzA5OTYwOTQsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo3NS45MzQ1MzIxNjU1MjczNCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NzMyMzY4NDY5MjM4MywiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjc3LjMyNDk4OTMxODg0NzY2LCJsZWZ0Ijo3Ljg2NTU0NTI3MjgyNzE0OCwid2lkdGgiOjQwLjY0OTM1MzAyNzM0Mzc1LCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6NzguNzE1NDQ2NDcyMTY3OTcsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6NDAuNjU0NjI0OTM4OTY0ODQ0LCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODAuMTEzNDEwOTQ5NzA3MDMsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6NDAuNjUxMDUwNTY3NjI2OTUsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo4MS41MDM4NjA0NzM2MzI4MSwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42NTQzNDI2NTEzNjcxOSwiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjgyLjg5NDMxNzYyNjk1MzEyLCJsZWZ0Ijo3Ljg2NTU0NTI3MjgyNzE0OCwid2lkdGgiOjQwLjMyMTgyMzEyMDExNzE5LCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODQuMjkyMjg5NzMzODg2NzIsImxlZnQiOjcuODY1NTQ1MjcyODI3MTQ4LCJ3aWR0aCI6MTUuODcwMjk1NTI0NTk3MTY4LCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODUuNjgyNzQ2ODg3MjA3MDMsImxlZnQiOjkuNTQ5NjQxNjA5MTkxODk1LCJ3aWR0aCI6MzguOTg5NzA0MTMyMDgwMDgsImhlaWdodCI6MS4yNTMyODA3NTg4NTc3Mjd9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjo4Ny4wNzMyMDQwNDA1MjczNCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC43MTg0OTQ0MTUyODMyLCJoZWlnaHQiOjEuMjUzMjgwNzU4ODU3NzI3fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6ODguNDcxMTY4NTE4MDY2NCwibGVmdCI6Ny44NjU1NDUyNzI4MjcxNDgsIndpZHRoIjo0MC42ODc0MTk4OTEzNTc0MiwiaGVpZ2h0IjoxLjI1MzI4MDc1ODg1NzcyN319LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjM0LjUxMzI3MTMzMTc4NzExLCJsZWZ0Ijo1MS45MzI3Nzc0MDQ3ODUxNTYsIndpZHRoIjo0MC42NDcwNTI3NjQ4OTI1OCwiaGVpZ2h0IjoyLjcxODY2NjU1MzQ5NzMxNDV9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjozOC43MjUxNTg2OTE0MDYyNSwibGVmdCI6NTEuOTMyNzc3NDA0Nzg1MTU2LCJ3aWR0aCI6NDAuNjQzMTM1MDcwODAwNzgsImhlaWdodCI6MS4yNTMyODg1MDc0NjE1NDc5fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6NDAuMTE1NjEyMDMwMDI5MywibGVmdCI6NTEuOTMyNzc3NDA0Nzg1MTU2LCJ3aWR0aCI6NDAuNjY0NTg1MTEzNTI1MzksImhlaWdodCI6MS4yNTMyODg1MDc0NjE1NDc5fX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNDMxIn19XQ%3D%3D. In integrated RAS for Atlantic salmon (Salmo salar), combined treatments yield 30% higher biomass densities without hypoxia-related stunting, as elevated DO supports mitochondrial ATP production and reduces lactate accumulation during peak feeding. Field applications in shrimp ponds demonstrate PAW-nanobubble infusions enhancing vibrio-resistant growth, with harvest sizes increasing by 18% alongside 12% FCR reductions. These physiological enhancements are frequently mediated by the upregulation of growth-related genes, such as growth hormone receptors and insulin-like growth factor, which drive anabolic processes and improve feed efficiency metrics (Elshafy et al., 2024; Zhao et al., 2022)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IldlaSIsImZhbWlseSI6IlpoYW8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiWGluIiwiZmFtaWx5IjoiQ3VpIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlppLVFpYW8iLCJmYW1pbHkiOiJXYW5nIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlJvbmciLCJmYW1pbHkiOiJZYW8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2hpLUh1YSIsImZhbWlseSI6IlhpZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJCYW95YW4iLCJmYW1pbHkiOiJHYW8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQ2hlbmd3dSIsImZhbWlseSI6IlpoYW5nIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkppbiIsImZhbWlseSI6Ik5pdSJ9fV0sImlkcyI6eyJpc3NuIjpbIjE2NjQtMzIyNCJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMzM4OS9maW1tdS4yMDIyLjk0MDkyOSIsInBtaWQiOiIzNTg2MDIzNCIsIm9wZW5hbGV4IjoiVzQyODM3OTc3NjkiLCJvcGVuQWNjZXNzSWQiOiIzNDM3MTk4MTU0OTAzNzExOTUxIn0sInRpdGxlIjoiQmVuZWZpY2lhbCBDaGFuZ2VzIGluIEdyb3d0aCBQZXJmb3JtYW5jZSwgQW50aW94aWRhbnQgQ2FwYWNpdHksIEltbXVuZSBSZXNwb25zZSwgSGVwYXRpYyBIZWFsdGgsIGFuZCBGbGVzaCBRdWFsaXR5IG9mIFRyYWNoaW5vdHVzIG92YXR1cyBGZWQgV2l0aCBPZWRvY2xhZGl1bSBjYXJvbGluaWFudW0iLCJhYnN0cmFjdCI6IlRoZSBwdXJwb3NlIG9mIHRoaXMgc3R1ZHkgaXMgdG8gYXNzZXNzIHRoZSBmZWFzaWJpbGl0eSBvZiBhc3RheGFudGhpbi1yaWNoIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjMzODkvZmltbXUuMjAyMi45NDA5MjkiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vd3d3LmZyb250aWVyc2luLm9yZy9hcnRpY2xlcy8xMC4zMzg5L2ZpbW11LjIwMjIuOTQwOTI5L3BkZiIsImlzc3VlZERhdGUiOiIyMDIyLTA3LTA0IiwiYWNjZXNzZWREYXRlIjoiMjAyNS0wOSIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiRnJvbnRpZXJzIGluIEltbXVub2xvZ3kiLCJ2b2x1bWUiOjEzLCJpbXBhY3RGYWN0b3IiOiI3LjI4In0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkZyb250aWVycyBNZWRpYSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NzIsImNpdGVkQnlDb3VudCI6MTV9fSwicXVvdGUiOiJOTywgYXMgYSBnYXNlb3VzIHNpZ25hbGluZyBtb2xlY3VsZSwgaXMgaW52b2x2ZWQgaW4gdGhlIHJlZ3VsYXRpb24gb2YgbmV1cm9uYWwgdHJhbnNtaXNzaW9uIGFuZCBhbnRpLWluZmxhbW1hdG9yeSwgYW50aXR1bW9yLCBhbmQgYW50aWJhY3RlcmlhbCBhY3Rpdml0aWVzICg2NCkuIE5PIGNhbiByZWFjdCB3aXRoIGZyZWUgcmFkaWNhbCBzdXBlcm94aWRlIHRvIGdlbmVyYXRlIGFjdGl2ZSBzdWJzdGFuY2VzLCBpbmNsdWRpbmcgbml0cm9nZW4gZGlveGlkZSwgZGluaXRyb2dlbiB0cmlveGlkZSwgYW5kIHBlcm94eW5pdHJpdGUsIHdoaWNoIGNhdXNlIHNldmVyZSBuaXRyb3NhdGlvbiBhbmQgb3hpZGF0aXZlIHN0cmVzcyB0byBiYWN0ZXJpYSwgZXZlbnR1YWxseSBkZXN0cm95aW5nIGNlbGwgbWVtYnJhbmVzIGFuZCBjYXVzaW5nIGNlbGwgZHlzZnVuY3Rpb24gb2YgYmFjdGVyaWEgKDY1KS4gVGhlcmVmb3JlLCBOTyBpcyBjb25zaWRlcmVkIHRvIGJlIGFuIGVmZmVjdGl2ZSBiYWN0ZXJpY2lkYWwgYWdlbnQgdGhhdCBraWxscyBicm9hZC1zcGVjdHJ1bSBiYWN0ZXJpYSwgZXNwZWNpYWxseSBkcnVnLXJlc2lzdGFudCBvbmVzICg2NikuIE5PIGlzIHByb2R1Y2VkIGJ5IHRoZSBOT1MtY2F0YWx5emVkIHJlYWN0aW9uIG9mIEwtYXJnaW5pbmUgd2l0aCBtb2xlY3VsYXIgb3h5Z2VuICg2NykuIFRoZXJlZm9yZSwgdGhlcmUgd2FzIGEgcG9zaXRpdmUgY29ycmVsYXRpb24gYmV0d2VlbiBOTyBjb250ZW50IGFuZCBOT1MgYWN0aXZpdHkuIE5PIGFuZCBOT1MgYXJlIGNvbnNpZGVyZWQgdG8gYmUgaW1wb3J0YW50IGFudGliYWN0ZXJpYWwgbW9sZWN1bGVzIGFnYWluc3QgcGF0aG9nZW4gaW5mZWN0aW9uIGluIGFxdWF0aWMgYW5pbWFscyAoNjgsIDY5KS4gSW4gdGhpcyBzdHVkeSwgVE5PUyBhY3Rpdml0eSBhbmQgTk8gY29udGVudCBpbmNyZWFzZWQgc2lnbmlmaWNhbnRseSBpbiB0aGUgbGl2ZXIgb2YgZmlzaCBmZWQgdGhlIE9DNSBkaWV0LiBUaGUgZmluZGluZ3Mgb2J0YWluZWQgaW4gdGhlIGN1cnJlbnQgc3R1ZHkgaW5kaWNhdGVkIHRoYXQgNSUgTy4gY2Fyb2xpbmlhbnVtIHBvd2RlciBwcm9tb3RlZCB0aGUgZGVmZW5zZSBhYmlsaXR5IG9mIFQuIG92YXR1cyBhZ2FpbnN0IHBhdGhvZ2VuaWMgaW5mZWN0aW9uLiBGYXR0eSBhY2lkIGNvbXBvc2l0aW9uIGlzIGFuIGltcG9ydGFudCBpbmRpY2F0b3IgcmVmbGVjdGluZyB0aGUgbnV0cml0aW9uYWwgdmFsdWUgb2YgZmlzaCwgd2hpY2ggaGFzIGFuIGltcGFjdCBvbiB0aGUgc2FsZXMgcHJpY2UuIEluIHRoZSBjdXJyZW50IHN0dWR5LCBmaXNoIGZlZCB0aGUgT0MwIG9yIE9DMSBkaWV0IHNob3dlZCBzaW1pbGFyIGZhdHR5IGFjaWQgcHJvZmlsZXMuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjo5LCJiYm94Ijp7InRvcCI6MzcuMzIyMDI5MTEzNzY5NTMsImxlZnQiOjUzLjQ0NzE4MTcwMTY2MDE1Niwid2lkdGgiOjM4LjkyMTA5NjgwMTc1NzgxLCJoZWlnaHQiOjEuMzQ1NDMwMzc0MTQ1NTA3OH19LHsicGFnZU51bWJlciI6OSwiYmJveCI6eyJ0b3AiOjM4Ljc5ODM0MzY1ODQ0NzI2NiwibGVmdCI6NTEuNDI4MjAzNTgyNzYzNjcsIndpZHRoIjo0MC45NDg1MjA2NjA0MDAzOSwiaGVpZ2h0IjoxLjM0NTQzMDM3NDE0NTUwNzh9fSx7InBhZ2VOdW1iZXIiOjksImJib3giOnsidG9wIjo0MC4yNzU2NTc2NTM4MDg1OTQsImxlZnQiOjUxLjQyODUyNzgzMjAzMTI1LCJ3aWR0aCI6NDAuOTQ4NTM1OTE5MTg5NDUsImhlaWdodCI6MS4zNDU0MzAzNzQxNDU1MDc4fX0seyJwYWdlTnVtYmVyIjo5LCJiYm94Ijp7InRvcCI6NDEuNzUxOTcyMTk4NDg2MzMsImxlZnQiOjUxLjQyODUyNzgzMjAzMTI1LCJ3aWR0aCI6NDAuOTQzMjU2Mzc4MTczODMsImhlaWdodCI6MS4zNDU0MzAzNzQxNDU1MDc4fX0seyJwYWdlTnVtYmVyIjo5LCJiYm94Ijp7InRvcCI6NDMuMjI4MjkwNTU3ODYxMzMsImxlZnQiOjUxLjQyODUyNzgzMjAzMTI1LCJ3aWR0aCI6NDAuOTQ5NTAxMDM3NTk3NjU2LCJoZWlnaHQiOjEuMzQ1NDMwMzc0MTQ1NTA3OH19LHsicGFnZU51bWJlciI6OSwiYmJveCI6eyJ0b3AiOjQ0LjY5NzQxNDM5ODE5MzM2LCJsZWZ0Ijo1MS40Mjg1Mjc4MzIwMzEyNSwid2lkdGgiOjQwLjk0MTY2NTY0OTQxNDA2LCJoZWlnaHQiOjEuMzQ1NDMwMzc0MTQ1NTA3OH19LHsicGFnZU51bWJlciI6OSwiYmJveCI6eyJ0b3AiOjQ2LjE3MzcyODk0Mjg3MTA5NCwibGVmdCI6NTEuNDI4NTI3ODMyMDMxMjUsIndpZHRoIjo0MC45NDIwMTI3ODY4NjUyMzQsImhlaWdodCI6MS4zNDU0MzAzNzQxNDU1MDc4fX0seyJwYWdlTnVtYmVyIjo5LCJiYm94Ijp7InRvcCI6NDcuNjUwMDQ3MzAyMjQ2MDk0LCJsZWZ0Ijo1MS40Mjg1Mjc4MzIwMzEyNSwid2lkdGgiOjQwLjk0NzQyOTY1Njk4MjQyLCJoZWlnaHQiOjEuMzQ1NDMwMzc0MTQ1NTA3OH19LHsicGFnZU51bWJlciI6OSwiYmJveCI6eyJ0b3AiOjQ5LjEyNjM2MTg0NjkyMzgzLCJsZWZ0Ijo1MS40Mjg1Mjc4MzIwMzEyNSwid2lkdGgiOjQwLjk0NjI2OTk4OTAxMzY3LCJoZWlnaHQiOjEuMzQ1NDMwMzc0MTQ1NTA3OH19LHsicGFnZU51bWJlciI6OSwiYmJveCI6eyJ0b3AiOjUwLjYwMjY4MDIwNjI5ODgzLCJsZWZ0Ijo1MS40Mjg1Mjc4MzIwMzEyNSwid2lkdGgiOjQwLjk0Njc4MTE1ODQ0NzI2NiwiaGVpZ2h0IjoxLjM0NTQzMDM3NDE0NTUwNzh9fSx7InBhZ2VOdW1iZXIiOjksImJib3giOnsidG9wIjo1Mi4wNzg5OTg1NjU2NzM4MywibGVmdCI6NTEuNDI4NTI3ODMyMDMxMjUsIndpZHRoIjo0MC45NTIzNDY4MDE3NTc4MSwiaGVpZ2h0IjoxLjM0NTQzMDM3NDE0NTUwNzh9fSx7InBhZ2VOdW1iZXIiOjksImJib3giOnsidG9wIjo1My41NDgxMjI0MDYwMDU4NiwibGVmdCI6NTEuNDI4NTI3ODMyMDMxMjUsIndpZHRoIjo0MC45NDk4MTM4NDI3NzM0NCwiaGVpZ2h0IjoxLjM0NTQzMDM3NDE0NTUwNzh9fSx7InBhZ2VOdW1iZXIiOjksImJib3giOnsidG9wIjo1NS4wMjQ0NDA3NjUzODA4NiwibGVmdCI6NTEuNDI4NTI3ODMyMDMxMjUsIndpZHRoIjo0MC45NDkwMTI3NTYzNDc2NTYsImhlaWdodCI6MS4zNDU0MzAzNzQxNDU1MDc4fX0seyJwYWdlTnVtYmVyIjo5LCJiYm94Ijp7InRvcCI6NTYuNTAwNzU1MzEwMDU4NTk0LCJsZWZ0Ijo1MS40Mjg1Mjc4MzIwMzEyNSwid2lkdGgiOjQwLjk0MjI2ODM3MTU4MjAzLCJoZWlnaHQiOjEuMzQ1NDMwMzc0MTQ1NTA3OH19LHsicGFnZU51bWJlciI6OSwiYmJveCI6eyJ0b3AiOjU3Ljk3NzA2OTg1NDczNjMzLCJsZWZ0Ijo1MS40Mjg1Mjc4MzIwMzEyNSwid2lkdGgiOjQwLjk0OTgzMjkxNjI1OTc2NiwiaGVpZ2h0IjoxLjM0NTQzMDM3NDE0NTUwNzh9fSx7InBhZ2VOdW1iZXIiOjksImJib3giOnsidG9wIjo1OS40NTMzODQzOTk0MTQwNiwibGVmdCI6NTEuNDI4NTE2Mzg3OTM5NDUsIndpZHRoIjo0MC45NDgxOTI1OTY0MzU1NSwiaGVpZ2h0IjoxLjM0NTQzMDM3NDE0NTUwNzh9fSx7InBhZ2VOdW1iZXIiOjksImJib3giOnsidG9wIjo2MC45Mjk3MDI3NTg3ODkwNiwibGVmdCI6NTEuNDI4NTA4NzU4NTQ0OTIsIndpZHRoIjo0MC45NTM3ODQ5NDI2MjY5NSwiaGVpZ2h0IjoxLjM0NTQzMDM3NDE0NTUwNzh9fSx7InBhZ2VOdW1iZXIiOjksImJib3giOnsidG9wIjo2Mi40MDYwMjExMTgxNjQwNiwibGVmdCI6NTEuNDI4NTA4NzU4NTQ0OTIsIndpZHRoIjo0MC45NTE4Njk5NjQ1OTk2MSwiaGVpZ2h0IjoxLjM0NTQzMDM3NDE0NTUwNzh9fSx7InBhZ2VOdW1iZXIiOjksImJib3giOnsidG9wIjo2My44NzUxNDQ5NTg0OTYwOTQsImxlZnQiOjUxLjQyODM0ODU0MTI1OTc2Niwid2lkdGgiOjE4LjQwNTE2MjgxMTI3OTI5NywiaGVpZ2h0IjoxLjM0NTQzMDM3NDE0NTUwNzh9fSx7InBhZ2VOdW1iZXIiOjksImJib3giOnsidG9wIjo2NS4zNTE0NjMzMTc4NzExLCJsZWZ0Ijo1My40NDczMjI4NDU0NTg5ODQsIndpZHRoIjozOC45MzMwNzQ5NTExNzE4NzUsImhlaWdodCI6MS4zNDU0MzAzNzQxNDU1MDc4fX0seyJwYWdlTnVtYmVyIjo5LCJiYm94Ijp7InRvcCI6NjYuODI3NzgxNjc3MjQ2MSwibGVmdCI6NTEuNDI4MTk5NzY4MDY2NDA2LCJ3aWR0aCI6NDAuOTUwNzQ0NjI4OTA2MjUsImhlaWdodCI6MS4zNDU0MjY0NDAyMzg5NTI2fX0seyJwYWdlTnVtYmVyIjo5LCJiYm94Ijp7InRvcCI6NjguMzA0MTAwMDM2NjIxMSwibGVmdCI6NTEuNDI4MTkyMTM4NjcxODc1LCJ3aWR0aCI6NDAuOTUyODQyNzEyNDAyMzQ0LCJoZWlnaHQiOjEuMzQ1NDI2NDQwMjM4OTUyNn19LHsicGFnZU51bWJlciI6OSwiYmJveCI6eyJ0b3AiOjY5Ljc4MDQxODM5NTk5NjEsImxlZnQiOjUxLjQyODE5MjEzODY3MTg3NSwid2lkdGgiOjQwLjk1MjY2NzIzNjMyODEyNSwiaGVpZ2h0IjoxLjM0NTQyNjQ0MDIzODk1MjZ9fV19LCJjaHVua0lkIjoiYmViNThkY2UtZWQ2NC00N2U1LTkyMTMtYmU2NjQwODVhMzRhIiwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiMTAifX0seyJvcmlnaW4iOjQsIndvcmsiOnsidHlwZSI6OCwic3VidHlwZSI6eyJvbmVvZktpbmQiOiJhcnRpY2xlVHlwZSIsImFydGljbGVUeXBlIjoxfSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTWFuYXIgQmFoYWEgQWJkIiwiZmFtaWx5IjoiRWxzaGFmeSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBc21hYSBJYnJhaGltIE1vaGFtZWQgQWJkIiwiZmFtaWx5IjoiRWwtTW9uZW0ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSWJyYWhpbSBNLiIsImZhbWlseSI6IktoYXR0YWIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2FmYWEgRS4iLCJmYW1pbHkiOiJBYmRvIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNhYnJlZW4gRS4iLCJmYW1pbHkiOiJGYWRsIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkdhbGFsIEFib3UiLCJmYW1pbHkiOiJLaGFkaWdhIn19XSwiaWRzIjp7Imlzc24iOlsiMDk2Ny02MTIwIiwiMTU3My0xNDNYIl0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDA3L3MxMDQ5OS0wMjQtMDE0NDgtdyIsIm9wZW5hbGV4IjoiVzQzOTI2MjA4OTYiLCJvcGVuQWNjZXNzSWQiOiIxMTU3Mjg5MTE4OTczODY3NjI3NSJ9LCJ0aXRsZSI6IkRpZXRhcnkgZmVlZCBuYW5vemVvbGl0ZSwgUGVkaW9jb2NjdXMsIGFuZCBtZWRpdW0tY2hhaW4gZmF0dHkgYWNpZCBlbmhhbmNlZCBncm93dGggcGVyZm9ybWFuY2UgYW5kIHRyYW5zY3JpcHRpb24gb2YgZ3Jvd3RoLXJlbGF0ZWQgZ2VuZSBvZiBOaWxlIHRpbGFwaWEgKE9yZW9jaHJvbWlzIG5pbG90aWN1cykiLCJhYnN0cmFjdCI6IkFic3RyYWN0IFRoZSBjdXJyZW50IHRyaWFsIHdhcyBkZXNpZ25lZCB0byBldmFsdWF0ZSB0aGUgcG9zaXRpdmUgaW1wYWN0cyBvZiBkaWZmZXJlbnQgZmVlZCBhZGRpdGl2ZXMgc2luZ2x5IG9yIGluIGNvbWJpbmF0aW9uIG9uIHRoZSBncm93dGggcGVyZm9ybWFuY2UsIG51dHJpdGlvbmFsIHBhcmFtZXRlcnMsIGludGVzdGluYWwgaGlzdG9sb2d5LCBhbmQgZ2VuZSBleHByZXNzaW9uIG9mIHNvbWUgZ3Jvd3RoIGFuZCBmYXQgbWV0YWJvbGlzbS1yZWxhdGVkIGdlbmVzIGluIHRoZSBsaXZlciB0aXNzdWUgb2YgTmlsZSB0aWxhcGlhLiBUaGUgZXhwZXJpbWVudGVkIGZpc2ggd2VyZSBhbGxvY2F0ZWQgaW50byBlaWdodCBncm91cHMgaW4gYSBnbGFzcyBhcXVhcml1bSAoMTAgZmlzaC9hcXVhcml1bSBpbiB0cmlwbGljYXRlKSB3aXRoIFBlZGlvY29jY3VzIGFjaWRpbGFjdGljaSAsIG5hbm96ZW9saXRlcywgYW5kL29yIG1lZGl1bS1jaGFpbiBmYXR0eSBhY2lkIGFkZGl0aXZlcy4gVGhlIHN0dWRpZWQgdHJlYXRtZW50cyB3ZXJlIGNvbnRyb2wgKCBUIDAgKSwgbmFub3plb2xpdGUgKCBUIDEgKSwgUGVkaW9jb2NjdXMgKCBUIDIgKSwgbWVkaXVtLWNoYWluIGZhdHR5IGFjaWQgKCBUIDMgKSwgbmFub3plb2xpdGUgKyBQZWRpb2NvY2N1cyAoIFQgNCApLCBuYW5vemVvbGl0ZSArIG1lZGl1bS1jaGFpbiBmYXR0eSBhY2lkICggVCA1ICksIFBlZGlvY29jY3VzICsgbWVkaXVtLWNoYWluIGZhdHR5IGFjaWQgKCBUIDYgKSwgYW5kIG5hbm96ZW9saXRlICsgUGVkaW9jb2NjdXMgKyBtZWRpdW0tY2hhaW4gZmF0dHkgYWNpZCAoIFQgNyApLiBUaGUgcmVzdWx0cyBvZiB0aGUgZ3Jvd3RoIGFuZCBudXRyaXRpb25hbCBwYXJhbWV0ZXJzIChpLmUuLCBmaW5hbCBib2R5IHdlaWdodCwgdG90YWwgd2VpZ2h0IGdhaW4sIGZlZWQgaW50YWtlLCBzcGVjaWZpYyBncm93dGggcmF0ZSwgZmVlZCBjb252ZXJzaW9uIHJhdGlvLCBhbmQgcHJvdGVpbiBlZmZpY2llbmN5IHJhdGlvKSBvZiB0aWxhcGlhLWZlZCBkaWV0cyBzdXBwbGVtZW50ZWQgd2l0aCBQZWRpb2NvY2N1cyAsIG5hbm96ZW9saXRlcywgYW5kIG1lZGl1bS1jaGFpbiBmYXR0eSBhY2lkcyBpbXByb3ZlZCwgYnV0IHRoZSBjb21iaW5hdGlvbiBvZiB0aGVzZSBhZGRpdGl2ZXMgd2FzIHNpZ25pZmljYW50bHkgbW9yZSBlZmZlY3RpdmUuIE1vcmVvdmVyLCBleHByZXNzaW9uIG9mIGdyb3d0aCBob3Jtb25lIHJlY2VwdG9yIDEgZ2VuZSB3YXMgdXByZWd1bGF0ZWQgKCBQIOKJpCAwLjA1KSBpbiBUIDcgZmlzaCB3aGVuIGNvbXBhcmVkIHdpdGggVCAwICwgb3RoZXIgZ3JvdXBzIHNob3dpbmcgaW50ZXJtZWRpYXRlIHZhbHVlcy4gRXhwcmVzc2lvbiBvZiBpbnN1bGluLWxpa2UgZ3Jvd3RoIGZhY3Rvci0xIHdhcyB1cHJlZ3VsYXRlZCAoIFAg4omkIDAuMDUpIGluIFQgNSAsIFQgNiAsIGFuZCBUIDcgZmlzaCB3aGVuIGNvbXBhcmVkIHdpdGggdGhlIG90aGVyIGdyb3Vwcy4gVGhlIGV4cHJlc3Npb24gb2YgdGhlIGZhdHR5IGFjaWQtYmluZGluZyBwcm90ZWluIHdhcyBoaWdoZXIgaW4gVCA2IGFuZCBUIDcgZmlzaCAoIFAg4omkIDAuMDUpIHdoZW4gY29tcGFyZWQgd2l0aCBUIDAgLCBUIDEgLCBhbmQgVCAzIGZpc2guIEluIGNvbmNsdXNpb24sIGNvbWJpbmVkIGFkZGl0aXZlcyBoYWQgc2lnbmlmaWNhbnQgZWZmZWN0cyBvbiBpbXByb3ZpbmcgZ3Jvd3RoIGFuZCByZWd1bGF0aW5nIGdyb3d0aC1yZWxhdGVkIGdlbmVzLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDA3L3MxMDQ5OS0wMjQtMDE0NDgtdyIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9saW5rLnNwcmluZ2VyLmNvbS9jb250ZW50L3BkZi8xMC4xMDA3L3MxMDQ5OS0wMjQtMDE0NDgtdy5wZGYiLCJpc3N1ZWREYXRlIjoiMjAyNC0wMy0wOSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkFxdWFjdWx0dXJlIEludGVybmF0aW9uYWwiLCJpc3N1ZSI6NSwidm9sdW1lIjozMiwiaW1wYWN0RmFjdG9yIjoiMy4yNSJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjU5MTEsImxhc3RQYWdlIjo1OTMzfX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlNwcmluZ2VyIFNjaWVuY2UrQnVzaW5lc3MgTWVkaWEifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjc3LCJjaXRlZEJ5Q291bnQiOjJ9fSwicXVvdGUiOiJGb2xsb3dpbmcgdGhlIGluY2x1c2lvbiBvZiBtb3JpbmdhIGxlYXZlcyBpbiB0aGUgZmVlZCwgZ3Jvd3RoIGhvcm1vbmUgcmVjZXB0b3IsIElHRiwgbGlwby1wcm90ZWluIGxpcGFzZSwgYW5kIGZhdHR5IGFjaWQgc3ludGhhc2UgbVJOQSBsZXZlbHMgaW5jcmVhc2VkIHJlbGF0aXZlbHksIHdoaWNoIHByb21vdGVkIHBlcmZvcm1hbmNlIGFuZCBpbmR1Y2VkIGh5cG9saXBpZGVtaWMgcmVzcG9uc2UgKEVsLUthc3NhcyBldCBhbC4gMjAyMCkuIENvbmNsdXNpb24gT3ZlcmFsbCwgaXQgd2FzIG9ic2VydmVkIHRoYXQgdGhlcmUgd2VyZSBtYW55IGRpZmZlcmVudCBiZW5lZmljaWFsIGVmZmVjdHMgcmVzdWx0aW5nIGZyb20gdGhlIGRpZmZlcmVudCBkaWV0YXJ5IGFkZGl0aXZlcyB1c2VkIGluIHRoaXMgdHJpYWwuIFBlZGlvY29jY3VzLCBuYW5vemVvbGl0ZXMsIGFuZCBtZWRpdW0tY2hhaW4gZmF0dHkgYWNpZCwgc2luZ2x5IG9yIGluIGNvbWJpbmF0aW9uLCBpbmR1Y2VkIHBvc2l0aXZlIGVmZmVjdHMgb24gZ3Jvd3RoIHBlcmZvcm1hbmNlLCAgZ3Jvd3RoIHJhdGVzLCBhbmQgZ2VuZSBleHByZXNzaW9uIG9mIHRoZSBncm93dGggYW5kIGZhdCBtZXRhYm9saXNtLXJlbGF0ZWQgZ2VuZXMgd2l0aCAgbm9ybWFsIGludGVzdGluYWwgYXJjaGl0ZWN0dXJlLiBIb3dldmVyLCBjb21iaW5lZCBhZGRpdGl2ZXMgKGkuZS4sIHRocmVlIHRyZWF0bWVudHMpIGhhZCBzaWctbmlmaWNhbnQgZWZmZWN0cyBvbiBpbXByb3ZpbmcgZ3Jvd3RoIGFuZCByZWd1bGF0aW5nIGdyb3d0aC1yZWxhdGVkIGdlbmVzLiBUaGlzIGNvbmNsdXNpb24gIHdhcyBjb25maXJtZWQgYnkgdGhlIHBvc2l0aXZlIGVmZmVjdHMgb2YgY29tYmluZWQgdHJlYXRtZW50IG9uIHRoZSBmaW5hbCBib2R5IHdlaWdodCwgdG90YWwgIHdlaWdodCBnYWluLCBmZWVkIGludGFrZSwgU0dSLCBGQ1IsIGFuZCBQRVIgYXMgZ3Jvd3RoIHBhcmFtZXRlcnMuIEluIGFkZGl0aW9uLCB1cHJlZ3UtbGF0aW9uIG9mIHRoZSBHSFIxIGFuZCBJR0YxIGdlbmVzLiBTaW5nbHksIHRoZSB0aHJlZSBhZGRpdGlvbnMgZGVtb25zdHJhdGVkIGludHJpZ3VpbmcgIHJlc3VsdHMsIG9mZmVyaW5nIGEgcG90ZW50aWFsbHkgZWZmZWN0aXZlIG1lYW5zIG9mIGVuaGFuY2luZyBhcXVhY3VsdHVyZSBncm93dGguIFN1cHBsZW1lbnRhcnkgSW5mb3JtYXRpb24gVGhlIG9ubGluZSB2ZXJzaW9uIGNvbnRhaW5zIHN1cHBsZW1lbnRhcnkgbWF0ZXJpYWwgYXZhaWxhYmxlIGF0IGh0dHBzOi8vIGRvaS4gb3JnLyAxMC4gMTAwNy8gczEwNDk5LSAwMjQtIDAxNDQ4LXcuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjguMjk1MDU0NDM1NzI5OTgsImxlZnQiOjEwLjY0NTE3NDAyNjQ4OTI1OCwid2lkdGgiOjc5LjI0MzkwNDExMzc2OTUzLCJoZWlnaHQiOjUuNjA2MDkxMDIyNDkxNDU1fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjE3Ljk3MDY1OTI1NTk4MTQ0NSwibGVmdCI6MTAuNjQ1MTc0MDI2NDg5MjU4LCJ3aWR0aCI6MTIuMzE1MTM0MDQ4NDYxOTE0LCJoZWlnaHQiOjIuMDQ1OTU5NzExMDc0ODI5fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjIxLjY3MjY0NzQ3NjE5NjI5LCJsZWZ0IjoxMC42NDUxNzQwMjY0ODkyNTgsIndpZHRoIjo3OS4yNTczMjQyMTg3NSwiaGVpZ2h0IjoxNy42OTUzMDg2ODUzMDI3MzR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6NDAuMjE3NDIyNDg1MzUxNTYsImxlZnQiOjEwLjY0NTE3MzA3MjgxNDk0MSwid2lkdGgiOjc5LjE2NDI2MDg2NDI1NzgxLCJoZWlnaHQiOjMuMjAwNTE1MjcwMjMzMTU0M319XX0sImNodW5rSWQiOiIyMmNjYTEwZC03ZDQ2LTQ0OTAtYmY2My1hMzk0YWYwMzgwZjIiLCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI1OTI3In19XQ%3D%3D. Elevated oxygen concentrations further enhance water quality by supporting nitrifying bacteria that are crucial for the nitrogen cycle in aquatic environments, where oxygen-dependent aerobic bacterial processes require the maintenance of high dissolved oxygen levels for efficient conversion of ammonia to nitrate (Patkaew et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNvbmd3dXQiLCJmYW1pbHkiOiJQYXRrYWV3In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNhdGFwb3JuIiwiZmFtaWx5IjoiRGlyZWtidXNhcmFrb0ttIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IklrdW8iLCJmYW1pbHkiOiJIxLFyb25vIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlN1d2l0IiwiZmFtaWx5IjoiV3V0aGlzdXRoaW1ldGhhdmVlIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNvcmF3aXQiLCJmYW1pbHkiOiJQb3d0b25nc29vayJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJDaGV0dHVwb24iLCJmYW1pbHkiOiJQb29sanVuIn19XSwiaWRzIjp7Imlzc24iOlsiMDk3Mi04OTg4IiwiMjIzMS0wOTE2Il0sImlzYm4iOltdLCJkb2kiOiIxMC4xNDIwMi92ZXR3b3JsZC4yMDI0LjUwLTU4IiwicG1pZCI6IjM4NDA2MzYxIiwib3BlbmFsZXgiOiJXNDM5MDY3MzA3NCIsIm9wZW5BY2Nlc3NJZCI6Ijc1MzcyODU5NjI5NDI0ODYzNDgifSwidGl0bGUiOiJFZmZlY3Qgb2Ygc3VwZXJzYXR1cmF0ZWQgZGlzc29sdmVkIG94eWdlbiBvbiBncm93dGgtLCBzdXJ2aXZhbC0sIGFuZCBpbW11bmUtcmVsYXRlZCBnZW5lIGV4cHJlc3Npb24gb2YgUGFjaWZpYyB3aGl0ZSBzaHJpbXAgKExpdG9wZW5hZXVzIHZhbm5hbWVpKSIsImFic3RyYWN0IjoiQmFja2dyb3VuZCBhbmQgQWltOiBPeHlnZW4gY29uY2VudHJhdGlvbiBpcyBhbiBlc3NlbnRpYWwgd2F0ZXIgcXVhbGl0eSBwYXJhbWV0ZXIgZm9yIGFxdWFjdWx0dXJlIHN5c3RlbXMuIFJlY2VudGx5LCBzdXBlcnNhdHVyYXRlZCBkaXNzb2x2ZWQgb3h5Z2VuIChETykgaGFzIGJlZW4gd2lkZWx5IHVzZWQgaW4gYXF1YWN1bHR1cmUgc3lzdGVtcyB0byBwcmV2ZW50IG94eWdlbiBkZXBsZXRpb247IGhvd2V2ZXIsIHRoZSBsb25nLXRlcm0gZWZmZWN0cyBvZiBzdXBlcnNhdHVyYXRlZCBveHlnZW4gZXhwb3N1cmUgb24gYXF1YXRpYyBhbmltYWxzIGhhdmUgbm90IGJlZW4gc3R1ZGllZC4gSW4gdGhpcyBzdHVkeSwgd2UgZXhhbWluZWQgdGhlIGVmZmVjdHMgb2Ygc3VwZXJzYXR1cmF0ZWQgRE8gb24gdGhlIGdyb3d0aCwgc3Vydml2YWwsIGFuZCBnZW5lIGV4cHJlc3Npb24gb2YgUGFjaWZpYyB3aGl0ZSBzaHJpbXAgKExpdG9wZW5hZXVzIHZhbm5hbWVpKS4gTWF0ZXJpYWxzIGFuZCBNZXRob2RzOiBTcGVjaWZpYyBwYXRob2dlbi1mcmVlIHNocmltcCB3aXRoIGEgYm9keSB3ZWlnaHQgb2YgOC4yMiDCsSAwLjAzIGcgd2VyZSByYW5kb21seSBhc3NpZ25lZCB0byB0d28gZ3JvdXBzIHdpdGggZm91ciByZXBsaWNhdGVzIGF0IGEgZGVuc2l0eSBvZiAxNSBzaHJpbXBzIHBlciB0YW5rLiBTaHJpbXAgd2VyZSBjdWx0aXZhdGVkIGluIHJlY2lyY3VsYXRpbmcgdGFua3MgY29udGFpbmluZyA1MCBMIG9mIDE1IHBwdCBzZWF3YXRlciBpbiBlYWNoIHJlcGxpY2F0ZS4gT3h5Z2VuIHdhcyBzdXBwbGllZCBhdCA1IG1nL0wgdG8gdGhlIGNvbnRyb2wgdGFua3MgdXNpbmcgYW4gYWlyIG1pY3JvYnViYmxlIGdlbmVyYXRvciBhbmQgYXQgMTUgbWcvTCB0byB0aGUgdHJlYXRtZW50IHRhbmtzIHVzaW5nIGEgcHVyZSBveHlnZW4gbWljcm9idWJibGUgZ2VuZXJhdG9yLiBTaHJpbXAgd2VyZSBmZWQgY29tbWVyY2lhbCBmZWVkIHBlbGxldHMgY29udGFpbmluZyAzOSUgcHJvdGVpbiBhdCA0JSBvZiB0aGVpciBib2R5IHdlaWdodCBwZXIgZGF5IGZvciAzMCBkYXlzLiBBdmVyYWdlIGRhaWx5IGdyb3d0aCAoQURHKSBhbmQgZmVlZCBjb252ZXJzaW9uIHJhdGlvIChGQ1IpIHdlcmUgZGV0ZXJtaW5lZCBvbiBkYXlzIDE1IGFuZCAzMC4gU2hyaW1wIG1vbHRpbmcgd2FzIG1lYXN1cmVkIGV2ZXJ5IGRheS4gSW5kaXZpZHVhbCBoZW1vbHltcGggc2FtcGxlcyB3ZXJlIG9idGFpbmVkIGFuZCBhbmFseXplZCBmb3IgdG90YWwgaGVtb2N5dGUgY291bnQsIGRpZmZlcmVudGlhbCBoZW1vY3l0ZSBjb3VudCwgYW5kIGV4cHJlc3Npb24gb2YgZ3Jvd3RoLSBhbmQgaW1tdW5lLXJlbGF0ZWQgZ2VuZXMgYXQgdGhlIGVuZCBvZiB0aGUgZXhwZXJpbWVudC4gUmVzdWx0czogTG9uZy10ZXJtIGV4cG9zdXJlIHRvIHN1cGVyc2F0dXJhdGVkIERPIHNpZ25pZmljYW50bHkgYWZmZWN0ZWQgc2hyaW1wIGdyb3d0aC4gQWZ0ZXIgMzAgZGF5cyBvZiBzdXBlcnNhdHVyYXRlZCBETyB0cmVhdG1lbnQsIHRoZSBmaW5hbCB3ZWlnaHQgYW5kIEFERyB3ZXJlIDE0LjczIMKxIDAuMTYgZyBhbmQgMC4yMiDCsSAwLjA0LCByZXNwZWN0aXZlbHkuIFNocmltcCB0cmVhdGVkIHdpdGggbm9ybWFsIGFlcmF0aW9uIHNob3dlZCBzaWduaWZpY2FudGx5IGxvd2VyIHdlaWdodCAoMTIuMTMgwrEgMC4xMyBnKSBhbmQgQURHICgwLjEzIMKxIDAuMDApIGNvbXBhcmVkIHdpdGggdGhlIGNvbnRyb2wgZ3JvdXAuIEZDUiB3YXMgMS41NSDCsSAwLjA0IGluIHRoZSB0cmVhdG1lbnQgZ3JvdXAgYW5kIDIuNTEgwrEgMC4wOSBpbiB0aGUgY29udHJvbCBncm91cC4gTm90YWJseSwgdGhlIHNocmltcCBtb2x0aW5nIGNvdW50IHdhcyAxLjU1LSBmb2xkIGhpZ2hlciBpbiB0aGUgc3VwZXJzYXR1cmF0ZWQgRE8gdHJlYXRtZW50IHRoYW4gaW4gdGhlIHN1cGVyc2F0dXJhdGVkIERPIHRyZWF0bWVudC4gVGhlIGV4cHJlc3Npb24gb2YgZ3Jvd3RoLXJlbGF0ZWQgZ2VuZXMsIHN1Y2ggYXMgYWxwaGEtYW15bGFzZSwgY2F0aGVwc2luIEwsIGFuZCBjaGl0b3RyaW9zaWRhc2UsIHdhcyAxLjQwLSwgMS40OC0sIGFuZCAxLjM1LWZvbGQgaGlnaGVyLCByZXNwZWN0aXZlbHksIGFmdGVyIHN1cGVyc2F0dXJhdGVkIERPIHRyZWF0bWVudC4gTW9yZW92ZXIsIHRoZSB0cmVhdG1lbnQgaW5jcmVhc2VkIHRoZSBleHByZXNzaW9uIG9mIGFudGktbGlwb3BvbHlzYWNjaGFyaWRlIGZhY3RvciwgY3J1c3RpbiwgcGVuYWVpZGluMywgYW5kIGhlYXQgc2hvY2sgcHJvdGVpbiA3MCBnZW5lcyBieSAxLjIzLSwgMi4wNy0sIDQuMjAtLCBhbmQgNjc5LjA0LWZvbGQsIHJlc3BlY3RpdmVseSwgY29tcGFyZWQgdG8gdGhlIGNvbnRyb2xzLiBDb25jbHVzaW9uOiBTdXBlcnNhdHVyYXRlZCBETyBpbmNyZWFzZWQgZ3Jvd3RoIGFuZCBBREcgcHJvZHVjdGlvbiBhbmQgZGVjcmVhc2VkIEZDUi4gRnVydGhlcm1vcmUsIGVuaGFuY2VkIGltbXVuZS1yZWxhdGVkIGdlbmUgZXhwcmVzc2lvbiBieSBzdXBlcnNhdHVyYXRlZCBETyBtYXkgaW1wcm92ZSBzaHJpbXAgaGVhbHRoIGFuZCByZWR1Y2UgZGlzZWFzZSByaXNrIGR1cmluZyBjdWx0aXZhdGlvbi4gS2V5d29yZHM6IGdlbmUgZXhwcmVzc2lvbiwgZ3Jvd3RoIHBlcmZvcm1hbmNlLCBtb2x0aW5nLCBwYWNpZmljIHdoaXRlIHNocmltcCwgc3VwZXJzYXR1cmF0ZWQgZGlzc29sdmVkIG94eWdlbi4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTQyMDIvdmV0d29ybGQuMjAyNC41MC01OCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cudmV0ZXJpbmFyeXdvcmxkLm9yZy9Wb2wuMTcvSmFudWFyeS0yMDI0LzYucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjQtMDEtMDEiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTEwIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJWZXRlcmluYXJ5IFdvcmxkIiwiaW1wYWN0RmFjdG9yIjoiMi41OSJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjUwLCJsYXN0UGFnZSI6NTh9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiVmV0ZXJpbmFyeSBXb3JsZCJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NTMsImNpdGVkQnlDb3VudCI6N319LCJxdW90ZSI6IlRoaXMgcmVzdWx0ICBjYW4gYmUgYXR0cmlidXRlZCB0byB0aGUgZWZmaWNpZW5jeSBvZiB0aGUgZmlsdHJhdGlvbiAgc3lzdGVtLCB3aGljaCBlZmZpY2llbnRseSByZW1vdmVzIHNvbGlkIHdhc3RlIHVzaW5nICBzeW50aGV0aWMgd29vbC4gVFNTIGNvbmNlbnRyYXRpb25zIG9mIGFwcHJveGltYXRlbHkgIDYuNzXigJMxMy4wMCBtZy9MIHdlcmUgb2JzZXJ2ZWQgaW4gYm90aCBncm91cHMuICBUeXBpY2FsbHksIFRTUyBpbiBhcXVhY3VsdHVyZSBwb25kcyBhcmUgcHJpbWFyaWx5ICB3YXRlcmJvcm5lIG9yZ2FuaWMgcGFydGljbGVzIHN1c3BlbmRlZCBpbiB3YXRlci4gVFNTICBkZWNvbXBvc2l0aW9uIHNob3dzIGEgaGlnaCBveHlnZW4gZGVtYW5kIGFuZCAgZGVjcmVhc2VzIHRoZSBETyBjb25jZW50cmF0aW9uIGluIGN1bHR1cmUgc3lzdGVtcyBbNDJdLiBUaGUgYXZlcmFnZSBjb25jZW50cmF0aW9ucyBvZiBhbW1vbmlhLCBuaXRyaXRlLCBhbmQgbml0cmF0ZSBkaWQgbm90IGRpZmZlciBzaWduaWZpY2FudGx5IGJldHdlZW4gdGhlIHRyZWF0bWVudCBhbmQgY29udHJvbCB0YW5rcyAoVGFibGUtNCkuIERPIHBsYXlzIGFuICBpbXBvcnRhbnQgcm9sZSBpbiBuaXRyaWZpY2F0aW9uLiBFbGV2YXRlZCBveHlnZW4gY29uLWNlbnRyYXRpb25zIGVuaGFuY2Ugd2F0ZXIgcXVhbGl0eSBieSBzdXBwb3J0aW5nIG5pdHJpLWZ5aW5nIGJhY3RlcmlhIHRoYXQgYXJlIGNydWNpYWwgZm9yIHRoZSBuaXRyb2dlbiBjeWNsZSAgaW4gYXF1YXRpYyBlbnZpcm9ubWVudHMuIE94eWdlbi1kZXBlbmRlbnQgYWVyb2JpYyAgYmFjdGVyaWFsIHByb2Nlc3NlcyByZXF1aXJlIHRoZSBtYWludGVuYW5jZSBvZiBoaWdoICBETyBjb25jZW50cmF0aW9ucyBmb3IgZWZmaWNpZW50IGNvbnZlcnNpb24gb2YgYW1tby1uaWEgdG8gbml0cmF0ZS4gTml0cmF0ZSwgYSBsZXNzIHRveGljIGZvcm0gb2Ygbml0cm9nZW4sICB3aGljaCBjYW4gYmUgYXNzaW1pbGF0ZWQgb3IgcmVtb3ZlZCBpbiB3YXRlciBjaGFuZ2VzLCAgdGh1cyBpbXByb3Zpbmcgd2F0ZXIgcXVhbGl0eS4gQ29uc2VxdWVudGx5LCBzaHJpbXBzICByZWFyZWQgdW5kZXIgaGlnaC1veHlnZW4gY29uY2VudHJhdGlvbnMgZXhoaWJpdGVkICBpbXByb3ZlZCBoZWFsdGgsIGxvd2VyIHN0cmVzcywgYW5kIGVuaGFuY2VkIGltbXVuZSAgcmVzcG9uc2VzLCBsZWFkaW5nIHRvIHN1cGVyaW9yIGdyb3d0aCBjb21wYXJlZCB0byAgdGhvc2UgcmVhcmVkIHVuZGVyIGxvdy1veHlnZW4gY29uY2VudHJhdGlvbnMuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6MTYuNjM0NTgyNTE5NTMxMjUsImxlZnQiOjUxLjUxMTQ4NjA1MzQ2NjgsIndpZHRoIjo0MC4xNjM5NjMzMTc4NzEwOTQsImhlaWdodCI6MS40NDczMDA2NzI1MzExMjh9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjoxOC4xMDQwNzI1NzA4MDA3OCwibGVmdCI6NTEuNTExNDg2MDUzNDY2OCwid2lkdGgiOjQwLjE2NDExOTcyMDQ1ODk4NCwiaGVpZ2h0IjoxLjQ0NzMwMDY3MjUzMTEyOH19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjE5LjU3MzU2MjYyMjA3MDMxMiwibGVmdCI6NTEuNTExNDg2MDUzNDY2OCwid2lkdGgiOjQwLjE2OTY4MTU0OTA3MjI2NiwiaGVpZ2h0IjoxLjQ0NzMwMDY3MjUzMTEyOH19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjIxLjA0MzA1MjY3MzMzOTg0NCwibGVmdCI6NTEuNTExNDg2MDUzNDY2OCwid2lkdGgiOjQwLjE2NzQ4MDQ2ODc1LCJoZWlnaHQiOjEuNDQ3MzAwNjcyNTMxMTI4fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6MjIuNTEyNTM3MDAyNTYzNDc3LCJsZWZ0Ijo1MS41MTE0ODYwNTM0NjY4LCJ3aWR0aCI6NDAuMTY0MzI1NzE0MTExMzMsImhlaWdodCI6MS40NDczMDA2NzI1MzExMjh9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjoyMy45ODIwMTk0MjQ0Mzg0NzcsImxlZnQiOjUxLjUxMTQ4NjA1MzQ2NjgsIndpZHRoIjo0MC4xNjkxMzk4NjIwNjA1NSwiaGVpZ2h0IjoxLjQ0NzMwMDY3MjUzMTEyOH19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjI1LjQ1MTUxMTM4MzA1NjY0LCJsZWZ0Ijo1MS41MTE0ODYwNTM0NjY4LCJ3aWR0aCI6NDAuMTY1NjIyNzExMTgxNjQsImhlaWdodCI6MS40NDczMDA2NzI1MzExMjh9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjoyNi45MjA5OTU3MTIyODAyNzMsImxlZnQiOjUxLjUxMTQ4NjA1MzQ2NjgsIndpZHRoIjo0MC4xNjM3ODAyMTI0MDIzNDQsImhlaWdodCI6MS40NDczMDA2NzI1MzExMjh9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjoyOC4zOTA0ODU3NjM1NDk4MDUsImxlZnQiOjUxLjUxMTQ4NjA1MzQ2NjgsIndpZHRoIjozOS43MDc0NjIzMTA3OTEwMTYsImhlaWdodCI6MS40NDczMDA2NzI1MzExMjh9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjoyOS44NTk5NzU4MTQ4MTkzMzYsImxlZnQiOjU1LjM3NDE3NjAyNTM5MDYyNSwid2lkdGgiOjM2LjMwMzg1NTg5NTk5NjA5NCwiaGVpZ2h0IjoxLjQ0NzMwMDY3MjUzMTEyOH19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjMxLjMyOTQ2Mzk1ODc0MDIzNCwibGVmdCI6NTEuNTExNDg2MDUzNDY2OCwid2lkdGgiOjQwLjE2NjUxOTE2NTAzOTA2LCJoZWlnaHQiOjIyLjAyMDEzMjA2NDgxOTMzNn19XX0sImNodW5rSWQiOiIwY2E4NjEwOC1hODAzLTRlZjYtYjI0ZS1mNmZlY2Q0NGZiNWEiLCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI1NSJ9fV0%3D.

Practical implementation involves strategic dosing protocols tailored to species and life stages: pulse infusions (every 7-14 days) for grow-out phases or continuous low-level metering in biofloc systems. Economic analyses project 20-40% feed cost savings, pivotal for commercial scalability. However, variability in PAW generation- influenced by plasma source voltage, gas composition, and exposure time-necessitates standardization. Establishing uniform production parameters is critical to ensure reproducibility of growth outcomes across different aquaculture facilities (Mozzon et al., 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1hc3NpbW8iLCJmYW1pbHkiOiJNb3p6b24ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUm9iZXJ0YSIsImZhbWlseSI6IkZvbGlnbmkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQ2luemlhIiwiZmFtaWx5IjoiTWFubm96emkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiRmlsaXBwbyIsImZhbWlseSI6IkdhbGRlbnppIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlJvbW9sbyIsImZhbWlseSI6IkxhdXJpdGEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2lsdmlhIiwiZmFtaWx5IjoiVGFwcGkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTWFyY28gRGFsbGEiLCJmYW1pbHkiOiJSb3NhIn19XSwiaWRzIjp7Imlzc24iOlsiMDk2My05OTY5IiwiMTg3My03MTQ1Il0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDE2L2ouZm9vZHJlcy4yMDIzLjExMzgyMyIsInBtaWQiOiIzODE2MzY4NiIsIm9wZW5hbGV4IjoiVzQzODkyOTg5OTgiLCJvcGVuQWNjZXNzSWQiOiI1MjQ3NTEyMDQzNjI2NjM2NjU4In0sInRpdGxlIjoiRWZmZWN0IG9mIHBsYXNtYS1hY3RpdmF0ZWQgd2F0ZXIgKFBBVykgc29ha2luZyBvbiB0aGUgbGlwaWQgb3hpZGF0aW9uIG9mIHNhcmRpbmUgKFNhcmRpbmEgcGlsY2hhcmR1cykgZmlsbGV0cyIsImFic3RyYWN0IjoiVGhlIGVmZmljYWN5IG9mIHBsYXNtYS1hY3RpdmF0ZWQgd2F0ZXIgKFBBVykgYXMgYSBjaGVtaWNhbC1mcmVlIGFuZCBlbnZpcm9ubWVudGFsbHkgZnJpZW5kbHkgcHJlc2VydmF0aXZlIGhhcyBiZWVuIGRvY3VtZW50ZWQgZm9yIGEgdmFyaWV0eSBvZiBmb29kcywgYnV0IHRoZSBvbnNldCBvZiBsaXBpZCBveGlkYXRpb24gaW5kdWNlZCBieSBwbGFzbWEtcmVhY3RpdmUgc3BlY2llcyBoYXMgYmVlbiBsZXNzIGV4dGVuc2l2ZWx5IHN0dWRpZWQuIEluIHRoaXMgd29yaywgZ2xvYmFsIGluZGljZXMgKHBlcm94aWRlIHZhbHVlLCBVViBzcGVjaWZpYyBhYnNvcmJhbmNlKSBhbmQgZGlyZWN0IGFuYWx5dGljYWwgZGV0ZXJtaW5hdGlvbnMgb2Ygdm9sYXRpbGUgYW5kIG5vbi12b2xhdGlsZSBveGlkYXRpb24gcHJvZHVjdHMgd2VyZSBwZXJmb3JtZWQgb24gc2FyZGluZSBsaXBpZHMgZXh0cmFjdGVkIGZyb20gZmlzaCBmaWxsZXRzIGltbWVyc2VkIGluIFBBVyAodHJlYXRtZW50cykgYW5kIGRpc3RpbGxlZCB3YXRlciAoY29udHJvbHMpIGZvciAxMC0zMCBtaW4uIEV2aWRlbmNlIG9mIFBBVy1pbmR1Y2VkIGxpcGlkIG94aWRhdGlvbiB3YXMgcHJvdmlkZWQgYnkgaGlnaGVyIFVWIHNwZWNpZmljIGFic29yYmFuY2VzIGFuZCBoaWdoZXIgbGV2ZWxzIG9mIEM1LUM5IHNlY29uZGFyeSB2b2xhdGlsZSBveGlkYXRpb24gcHJvZHVjdHMgaW4gdGhlIHRyZWF0ZWQgc2FtcGxlcy4gSG93ZXZlciwgdGhlIGRlZ3JlZSBvZiBmYXR0eSBhY2lkIG94aWRhdGlvbiB3YXMgbm90IHN1ZmZpY2llbnQgdG8gY2F1c2UgYSBzaWduaWZpY2FudCByZWR1Y3Rpb24gaW4gbnV0cml0aW9uYWxseSB2YWx1YWJsZSBlaWNvc2FwZW50YWVub2ljIGFjaWQgYW5kIGRvY29zYWhleGFlbm9pYyBhY2lkLiBUd2VsdmUgY2hvbGVzdGVyb2wgb3hpZGF0aW9uIHByb2R1Y3RzIChDT1BzKSB3ZXJlIGlkZW50aWZpZWQgaW4gdGhlIHNhcmRpbmUgbGlwaWRzLCBidXQgbm8gc2lnbmlmaWNhbnQgZGlmZmVyZW5jZXMgaW4gdG90YWwgQ09QcyBjb250ZW50IHdlcmUgZm91bmQgYmV0d2VlbiBQQVcgcHJvY2Vzc2VkIGFuZCBjb250cm9sIHNhbXBsZXMuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjEwMTYvai5mb29kcmVzLjIwMjMuMTEzODIzIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAxNi9qLmZvb2RyZXMuMjAyMy4xMTM4MjMiLCJpc3N1ZWREYXRlIjoiMjAyMy0xMi0wNCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDciLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZvb2QgUmVzZWFyY2ggSW50ZXJuYXRpb25hbCIsInZvbHVtZSI6MTc2LCJpbXBhY3RGYWN0b3IiOiI4LjI3In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTEzODIzLCJsYXN0UGFnZSI6MTEzODIzfX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkVsc2V2aWVyIEJWIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo0MiwiY2l0ZWRCeUNvdW50Ijo3fX0sInF1b3RlIjoiSG93ZXZlciwgc2V2ZXJhbCAgcmVwb3J0cyBoYXZlIGVtcGhhc2lzZWQgdGhlIHJvbGUgb2Ygc2hvcnQtbGl2ZWQgc3BlY2llcyBwZXJveHluaXRyb3VzICBhY2lkL3Blcm94eW5pdHJpdGUsIHN1cGVyb3hpZGUgYW5kIGh5ZHJveHlsIHJhZGljYWxzLCBhbmQgc2luZ2xldCBveHktZ2VuIChQZXJpbmJhbiwgT3JzYXQsICYgUmFnaGF2YW4sIDIwMTkpLiBJdCB3YXMgZm91bmQgdGhhdCBzb2FraW5nIHdpdGggIFBBVyB3YXMgc3VjY2Vzc2Z1bCBpbiBpbmFjdGl2YXRpbmcgU2hld2FuZWxsYSBwdXRyZWZhY2llbnMgaW4gZmlsbGV0cyBvZiAgWWVsbG93IFJpdmVyIGNhcnAgKEN5cHJpbnVzIGNhcnBpbykgKExpdSwgWmhhbmcsIE1lbmcsIEJhaSwgJiBEb25nLCAgMjAyMSkgYW5kIEwuIG1vbm9jeXRvZ2VuZXMgYW5kIFMuIFR5cGhpbXVyaXVtIGluIGdyYXNzIGNhcnAgKEN0ZW5vLXBoYXJ5bmdvZG9uIGlkZWxsYSkgKEVzdWEsIENoZW5nLCAmIFN1biwgMjAyMCksIGFuZCBleHRlbmRpbmcgdGhlIHNoZWxmICBsaWZlIG9mIEFzaWFuIHNlYSBiYXNzIChMYXRlcyBjYWxjYXJpZmVyKSBzdGVha3MgKENoYWlqYW4gZXQgYWwuLCAyMDIxKS4gUEFXICBpY2UgKExpYW8gZXQgYWwuLCAyMDE4KSBhbmQgUEFXIGdsYXppbmcgKEhlcmlhbnRvIGV0IGFsLiwgMjAyMikgaGF2ZSBiZWVuICBzdWdnZXN0ZWQgYXMgcG9zc2libGUgcHJlc2VydmF0aW9uIG1ldGhvZHMgZm9yIGZyZXNoIHNocmltcHMsIHdoaWxlICBaaGFvIGV0IGFsLiAoMjAyMSkgaW52ZXN0aWdhdGVkIHRoZSBkZWNvbnRhbWluYXRpbmcgZWZmZWN0IG9mIGEgY29tYmktbmF0aW9uIG9mIFBBVyBkaXBwaW5nIGFuZCBzb25pY2F0aW9uIG9uIG1hY2tlcmVsIGZpbGxldHMuICAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjY5LjYwNTYxMzcwODQ5NjEsImxlZnQiOjYuMzE0MjgwNTA5OTQ4NzMwNSwid2lkdGgiOjQyLjU2Njc5OTE2MzgxODM2LCJoZWlnaHQiOjEuNjQyNTY0MTc3NTEzMTIyNn19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjcwLjkxOTg2ODQ2OTIzODI4LCJsZWZ0Ijo2LjMxNDI4MDUwOTk0ODczMDUsIndpZHRoIjo0Mi41NjkyMTc2ODE4ODQ3NjYsImhlaWdodCI6MS42NDI1NjQxNzc1MTMxMjI2fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NzIuMjQxMzYzNTI1MzkwNjIsImxlZnQiOjYuMzE0MjgwNTA5OTQ4NzMwNSwid2lkdGgiOjQyLjE3NzEyNDAyMzQzNzUsImhlaWdodCI6MS42NDI1NjQxNzc1MTMxMjI2fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NzMuNTU1NjEwNjU2NzM4MjgsImxlZnQiOjYuMzE0MjgwNTA5OTQ4NzMwNSwid2lkdGgiOjQyLjU3NDI5NTA0Mzk0NTMxLCJoZWlnaHQiOjEuNjQyNTY0MTc3NTEzMTIyNn19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjc0Ljg3Njk5MTI3MTk3MjY2LCJsZWZ0Ijo2LjMxNDI4MDUwOTk0ODczMDUsIndpZHRoIjo0Mi41NzU2Mzc4MTczODI4MSwiaGVpZ2h0IjoxLjY0MjU2NDE3NzUxMzEyMjZ9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo3Ni4xOTEyNTM2NjIxMDkzOCwibGVmdCI6Ni4zMTQyODA1MDk5NDg3MzA1LCJ3aWR0aCI6NDIuNTc0Njk5NDAxODU1NDcsImhlaWdodCI6MS42NDI1NjQxNzc1MTMxMjI2fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NzcuNTEyNzMzNDU5NDcyNjYsImxlZnQiOjYuMzE0MjgwNTA5OTQ4NzMwNSwid2lkdGgiOjQyLjE4MTMwNDkzMTY0MDYyNSwiaGVpZ2h0IjoxLjY0MjU2NDE3NzUxMzEyMjZ9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo3OC44MjY5OTU4NDk2MDkzOCwibGVmdCI6Ni4zMTQyODA1MDk5NDg3MzA1LCJ3aWR0aCI6NDIuNTczODk0NTAwNzMyNDIsImhlaWdodCI6MS42NDI1NjQxNzc1MTMxMjI2fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6ODAuMTQ4Mzg0MDk0MjM4MjgsImxlZnQiOjYuMzE0MjgwNTA5OTQ4NzMwNSwid2lkdGgiOjQyLjU3MzA4OTU5OTYwOTM3NSwiaGVpZ2h0IjoxLjY0MjU2NDE3NzUxMzEyMjZ9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo4MS40NjI2NDY0ODQzNzUsImxlZnQiOjYuMzE0MjgwNTA5OTQ4NzMwNSwid2lkdGgiOjQyLjU2ODAwODQyMjg1MTU2LCJoZWlnaHQiOjEuNjQyNTY0MTc3NTEzMTIyNn19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjgyLjc4NDExODY1MjM0Mzc1LCJsZWZ0Ijo2LjMxNDI4MDUwOTk0ODczMDUsIndpZHRoIjo0Mi41NzI4MjYzODU0OTgwNSwiaGVpZ2h0IjoxLjY0MjU2NDE3NzUxMzEyMjZ9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo4NC4wOTgzODEwNDI0ODA0NywibGVmdCI6Ni4zMTQyODA1MDk5NDg3MzA1LCJ3aWR0aCI6NDIuMTgxMTQwODk5NjU4MiwiaGVpZ2h0IjoxLjY0MjU2NDE3NzUxMzEyMjZ9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo4NS40MTk3NjkyODcxMDkzOCwibGVmdCI6Ni4zMTQyODA1MDk5NDg3MzA1LCJ3aWR0aCI6MzQuODE5ODQzMjkyMjM2MzMsImhlaWdodCI6MS42NDI1NjQxNzc1MTMxMjI2fX1dfSwiY2h1bmtJZCI6Ijg1OWZmNzQzLWRlYWQtNDc5Yy05ODkxLTg3YjA1NmJjMDMzNCIsImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjExMzgyNCJ9fV0%3D.

Despite these advances, challenges include optimizing species-specific thresholds to prevent nitrate bioaccumulation in filter feeders and validating long-term performance under fluctuating salinities. Future research should prioritize genomic profiling of growth trajectories and multi-omics integration to refine formulations, positioning PAW as a cornerstone for sustainable, high-yield aquaculture.

Challenges and Future Perspectives

While both nanobubble and plasma water technologies show great promise for use in aquaculture, there are multiple challenges that must be overcome before they can see widespread implementation. A consolidated overview of the major challenges and the corresponding future research directions is presented in Table 5. More studies are needed to explore the potential impacts of these technologies on aquatic life and the environment, and how much leeway they may have. Nanobubble and plasma water generation systems must be healthily scalable and cost-effective for sustainable aquaculture to prosper commercially. Specifically, standardizing treatment parameters such as exposure time, plasma source type, and gas composition across different species and developmental stages is crucial for achieving reproducible results (Kumar, 2025)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlB1bml0IiwiZmFtaWx5IjoiS3VtYXIifX1dLCJpZHMiOnsiaXNzbiI6WyIyMzIxLTk2NTMiXSwiaXNibiI6W10sImRvaSI6IjEwLjIyMjE0L2lqcmFzZXQuMjAyNS43MzE1MiIsIm9wZW5hbGV4IjoiVzQ0MTMwOTM1MTgiLCJvcGVuQWNjZXNzSWQiOiI0MTA4ODcxODU1ODkxOTEyNjEwIn0sInRpdGxlIjoiQmlvc3RpbXVsYXRvcnkgRWZmZWN0cyBvZiBQbGFzbWEgb24gU2VlZHMgYW5kIFBsYW50cyIsImFic3RyYWN0IjoiTm9udGhlcm1hbCBwbGFzbWEgdHJlYXRtZW50cyBpbmNsdWRpbmcgc2VlZCBwcmltaW5nLCBwbGFzbWEtYWN0aXZhdGVkIHdhdGVyIChQQVcpLCBhbmQgZGlyZWN0IHBsYW50IHN0aW11bGF0aW9uIGhhdmUgZ2FpbmVkIHJlY29nbml0aW9uIGFzIGlubm92YXRpdmUsIGVjby1mcmllbmRseSBzdHJhdGVnaWVzIHRvIGJvb3N0IHNlZWQgZ2VybWluYXRpb24sIHBsYW50IGdyb3d0aCwgYmlvc3RpbXVsYXRpb24sIGFuZCBjcm9wIHlpZWxkLiBUaGlzIHBhcGVyIHJldmlld3MgcmVjZW50IGFkdmFuY2VtZW50cyBpbiBwbGFzbWEtYmFzZWQgc2VlZCB0cmVhdG1lbnQgYW5kIGlycmlnYXRpb24gdGVjaG5pcXVlcywgZXhhbWluaW5nIHRoZWlyIHBoeXNpb2xvZ2ljYWwsIGJpb2NoZW1pY2FsLCBhbmQgbW9sZWN1bGFyIGltcGFjdHMgb24gdmFyaW91cyBjcm9wcy4gSXQgYWxzbyBleHBsb3JlcyB0aGUgcm9sZSBvZiByZWFjdGl2ZSBzcGVjaWVzLCBjaGFuZ2VzIGluIGdlbmUgZXhwcmVzc2lvbiwgYW5kIGltcHJvdmVkIG51dHJpZW50IHVwdGFrZSByZXN1bHRpbmcgZnJvbSBwbGFzbWEgZXhwb3N1cmUuIEtleSBjYXNlIHN0dWRpZXMgYXJlIHByZXNlbnRlZCB0byBpbGx1c3RyYXRlIHByYWN0aWNhbCBhcHBsaWNhdGlvbnMsIHdoaWxlIGF0dGVudGlvbiBpcyBnaXZlbiB0byB0aGUgaW5mbHVlbmNlIG9mIHBsYXNtYSBkZXZpY2UgdHlwZXMsIHRyZWF0bWVudCBwYXJhbWV0ZXJzLCBhbmQgaW50ZWdyYXRpb24gd2l0aCBvdGhlciBzdXN0YWluYWJsZSB0ZWNobm9sb2dpZXMuIFRoZSBwYXBlciBjb25jbHVkZXMgYnkgYWRkcmVzc2luZyB0ZWNobm8tZWNvbm9taWMgZmVhc2liaWxpdHksIGZpZWxkLXNjYWxlIGFkYXB0YWJpbGl0eSwgYW5kIG91dGxpbmluZyBjcml0aWNhbCBkaXJlY3Rpb25zIGZvciBmdXR1cmUgcmVzZWFyY2ggaW4gcGxhc21hLWFzc2lzdGVkIGFncmljdWx0dXJlIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjIyMjE0L2lqcmFzZXQuMjAyNS43MzE1MiIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjIyMjE0L2lqcmFzZXQuMjAyNS43MzE1MiIsImlzc3VlZERhdGUiOiIyMDI1LTA3LTE2IiwiYWNjZXNzZWREYXRlIjoiMjAyNS0wOSIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiSW50ZXJuYXRpb25hbCBKb3VybmFsIGZvciBSZXNlYXJjaCBpbiBBcHBsaWVkIFNjaWVuY2UgYW5kIEVuZ2luZWVyaW5nIFRlY2hub2xvZ3kiLCJpc3N1ZSI6Nywidm9sdW1lIjoxMywiaW1wYWN0RmFjdG9yIjoiMC40MSJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjExMDEsImxhc3RQYWdlIjoxMTEyfX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkludGVybmF0aW9uYWwgSm91cm5hbCBmb3IgUmVzZWFyY2ggaW4gQXBwbGllZCBTY2llbmNlIGFuZCBFbmdpbmVlcmluZyBUZWNobm9sb2d5IChJSlJBU0VUKSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MCwiY2l0ZWRCeUNvdW50IjowfX0sInF1b3RlIjoiUEFXIHJldGFpbnMgYW50aW1pY3JvYmlhbCBhbmQgZ3Jvd3RoLXByb21vdGluZyBwcm9wZXJ0aWVzLCBtYWtpbmcgaXQgc3VpdGFibGUgZm9yIGlycmlnYXRpb24sIGZvbGlhciBzcHJheSwgYW5kICBwb3N0LWhhcnZlc3QgY2xlYW5pbmcuIEl0IHByb3ZpZGVzIGEgbm9uLXRveGljLCByZXNpZHVlLWZyZWUgYWx0ZXJuYXRpdmUgdG8gY2hlbWljYWwgZnVuZ2ljaWRlcyBhbmQgZmVydGlsaXplcnMsIGVuaGFuY2luZyB0aGUgIGVjb2xvZ2ljYWwgZm9vdHByaW50IG9mIGFncmljdWx0dXJhbCBvcGVyYXRpb25zLiAgRGVzcGl0ZSB0aGVzZSBlbmNvdXJhZ2luZyBkZXZlbG9wbWVudHMsIHNldmVyYWwgY2hhbGxlbmdlcyBtdXN0IGJlIGFkZHJlc3NlZCBmb3IgcGxhc21hIHRvIGJlIHdpZGVseSBhZG9wdGVkLiBTdGFuZGFyZGl6aW5nICB0cmVhdG1lbnQgcGFyYW1ldGVycywgc3VjaCBhcyBleHBvc3VyZSB0aW1lLCBwbGFzbWEgc291cmNlIHR5cGUsIGFuZCBnYXMgY29tcG9zaXRpb24gYWNyb3NzIGRpZmZlcmVudCBjcm9wIHNwZWNpZXMgYW5kICBkZXZlbG9wbWVudGFsIHN0YWdlcyBpcyBjcnVjaWFsIGZvciBhY2hpZXZpbmcgcmVwcm9kdWNpYmxlIHJlc3VsdHMuIEFkZGl0aW9uYWxseSwgc2NhbGluZyB1cCBwbGFzbWEgdGVjaG5vbG9naWVzIGZvciBmaWVsZC1sZXZlbCAgZGVwbG95bWVudCByZXF1aXJlcyB0aGUgZGVzaWduIG9mIGNvc3QtZWZmZWN0aXZlLCBlbmVyZ3ktZWZmaWNpZW50IHN5c3RlbXMgdGhhdCBpbnRlZ3JhdGUgc2VhbWxlc3NseSB3aXRoIGV4aXN0aW5nIGFncmljdWx0dXJhbCAgaW5mcmFzdHJ1Y3R1cmUuICBUaGVyZSBhbHNvIHJlbWFpbnMgYSBjcml0aWNhbCBuZWVkIHRvIGRlZXBlbiBvdXIgbWVjaGFuaXN0aWMgdW5kZXJzdGFuZGluZyBvZiBwbGFzbWHigJNwbGFudCBpbnRlcmFjdGlvbnMgYXQgdGhlIG1vbGVjdWxhciwgZ2VuZXRpYywgIGFuZCBlcGlnZW5ldGljIGxldmVscy4gTG9uZy10ZXJtIHN0dWRpZXMgZXhhbWluaW5nIHRoZSBoZXJpdGFiaWxpdHkgb2YgcGxhc21hLWluZHVjZWQgdHJhaXRzLCBwb3NzaWJsZSBtdXRhZ2VuaWMgZWZmZWN0cywgYW5kICBlY29sb2dpY2FsIGNvbnNlcXVlbmNlcyBhcmUgZXNzZW50aWFsIGZvciBkZXZlbG9waW5nIHNhZmUgcmVndWxhdG9yeSBmcmFtZXdvcmtzIGFuZCBlbnN1cmluZyBwdWJsaWMgYWNjZXB0YW5jZS4gIE5ldmVydGhlbGVzcywgdGhlIGN1bXVsYXRpdmUgYm9keSBvZiBleHBlcmltZW50YWwgZXZpZGVuY2Ugc3Ryb25nbHkgYWR2b2NhdGVzIGZvciBzaWduaWZpY2FudCBpbnZlc3RtZW50IGluIHBsYXNtYS1iYXNlZCAgYWdyaWN1bHR1cmFsIHRlY2hub2xvZ2llcy4gTnVtZXJvdXMgc3R1ZGllcyBoYXZlIGRlbW9uc3RyYXRlZCBwbGFzbWHigJlzIGNhcGFiaWxpdHkgdG8gc2ltdWx0YW5lb3VzbHkgZW5oYW5jZSBnZXJtaW5hdGlvbiwgZ3Jvd3RoLCAgYW5kIHByb3RlY3Rpb24gd2l0aG91dCBpbnRyb2R1Y2luZyBoYXJtZnVsIHJlc2lkdWVzIG9yIGV4Y2Vzc2l2ZSBlbmVyZ3kgZGVtYW5kcy4gSXQgcHJlc2VudHMgYW4gaWRlYWwgaW50ZXJzZWN0aW9uIGJldHdlZW4gIHNjaWVudGlmaWMgaW5ub3ZhdGlvbiBhbmQgZWNvbG9naWNhbCByZXNwb25zaWJpbGl0eS4gIFRodXMsIHBsYXNtYSB0ZWNobm9sb2d5IG9mZmVycyBhIHRyYW5zZm9ybWF0aXZlIGFwcHJvYWNoIHRvIGNvbnRlbXBvcmFyeSBhZ3JpY3VsdHVyZS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjExLCJiYm94Ijp7InRvcCI6MjYuMjM1NTc2NjI5NjM4NjcyLCJsZWZ0Ijo2LjU4ODIzNTM3ODI2NTM4MSwid2lkdGgiOjg3LjIyOTg1ODM5ODQzNzUsImhlaWdodCI6MS42OTc4MTc2ODMyMTk5MDk3fX0seyJwYWdlTnVtYmVyIjoxMSwiYmJveCI6eyJ0b3AiOjI3LjkzMjU0ODUyMjk0OTIyLCJsZWZ0Ijo2LjU4ODIzNTM3ODI2NTM4MSwid2lkdGgiOjg3LjIzMjAwMjI1ODMwMDc4LCJoZWlnaHQiOjEuNjk3ODE3NjgzMjE5OTA5N319LHsicGFnZU51bWJlciI6MTEsImJib3giOnsidG9wIjoyOS41OTkyMDY5MjQ0Mzg0NzcsImxlZnQiOjYuNTg4MjM1Mzc4MjY1MzgxLCJ3aWR0aCI6MzAuNTY3ODQ4MjA1NTY2NDA2LCJoZWlnaHQiOjEuNjk3ODE3NjgzMjE5OTA5N319LHsicGFnZU51bWJlciI6MTEsImJib3giOnsidG9wIjozMS4yNjU4Nzg2NzczNjgxNjQsImxlZnQiOjYuNTg4MjM1Mzc4MjY1MzgxLCJ3aWR0aCI6ODcuMjMzMDA5MzM4Mzc4OSwiaGVpZ2h0IjoxLjY5NzgxOTcwOTc3NzgzMn19LHsicGFnZU51bWJlciI6MTEsImJib3giOnsidG9wIjozMi45MzI1NDg1MjI5NDkyMiwibGVmdCI6Ni41ODgyMzUzNzgyNjUzODEsIndpZHRoIjo4Ny4yNDA3Mzc5MTUwMzkwNiwiaGVpZ2h0IjoxLjY5NzgxNzY4MzIxOTkwOTd9fSx7InBhZ2VOdW1iZXIiOjExLCJiYm94Ijp7InRvcCI6MzQuNTk5MjE2NDYxMTgxNjQsImxlZnQiOjYuNTg4MjM1Mzc4MjY1MzgxLCJ3aWR0aCI6ODcuMjM3NDExNDk5MDIzNDQsImhlaWdodCI6MS42OTc4MTc2ODMyMTk5MDk3fX0seyJwYWdlTnVtYmVyIjoxMSwiYmJveCI6eyJ0b3AiOjM2LjI2NTg4MDU4NDcxNjgsImxlZnQiOjYuNTg4MjM1Mzc4MjY1MzgxLCJ3aWR0aCI6ODcuMjMzMzM3NDAyMzQzNzUsImhlaWdodCI6MS42OTc4MTc2ODMyMTk5MDk3fX0seyJwYWdlTnVtYmVyIjoxMSwiYmJveCI6eyJ0b3AiOjM3LjkzMjU0ODUyMjk0OTIyLCJsZWZ0Ijo2LjU4ODIzNTM3ODI2NTM4MSwid2lkdGgiOjkuNjI2NjY4OTMwMDUzNzExLCJoZWlnaHQiOjEuNjk3ODE3NjgzMjE5OTA5N319LHsicGFnZU51bWJlciI6MTEsImJib3giOnsidG9wIjozOS41OTkyMTI2NDY0ODQzNzUsImxlZnQiOjYuNTg4MjM1Mzc4MjY1MzgxLCJ3aWR0aCI6ODcuMjM0NTI3NTg3ODkwNjIsImhlaWdodCI6MS42OTc4MTc2ODMyMTk5MDk3fX0seyJwYWdlTnVtYmVyIjoxMSwiYmJveCI6eyJ0b3AiOjQxLjI2NTg4MDU4NDcxNjgsImxlZnQiOjYuNTg4MjM1Mzc4MjY1MzgxLCJ3aWR0aCI6ODcuMjI0NjU1MTUxMzY3MTksImhlaWdodCI6MS42OTc4MTc2ODMyMTk5MDk3fX0seyJwYWdlTnVtYmVyIjoxMSwiYmJveCI6eyJ0b3AiOjQyLjkzMjU0ODUyMjk0OTIyLCJsZWZ0Ijo2LjU4ODIzNTM3ODI2NTM4MSwid2lkdGgiOjc0LjU2Nzg0MDU3NjE3MTg4LCJoZWlnaHQiOjEuNjk3ODE3NjgzMjE5OTA5N319LHsicGFnZU51bWJlciI6MTEsImJib3giOnsidG9wIjo0NC42Mjk1MTI3ODY4NjUyMzQsImxlZnQiOjYuNTg4MjM1Mzc4MjY1MzgxLCJ3aWR0aCI6ODcuMjM1NTcyODE0OTQxNCwiaGVpZ2h0IjoxLjY5NzgxNzY4MzIxOTkwOTd9fSx7InBhZ2VOdW1iZXIiOjExLCJiYm94Ijp7InRvcCI6NDYuMjk2MTgwNzI1MDk3NjU2LCJsZWZ0Ijo2LjU4ODIzNTM3ODI2NTM4MSwid2lkdGgiOjg3LjIzNTQ2NjAwMzQxNzk3LCJoZWlnaHQiOjEuNjk3ODE3NjgzMjE5OTA5N319LHsicGFnZU51bWJlciI6MTEsImJib3giOnsidG9wIjo0Ny45NjI4NDg2NjMzMzAwOCwibGVmdCI6Ni41ODgyMzUzNzgyNjUzODEsIndpZHRoIjo4Ny4yMzYyMTM2ODQwODIwMywiaGVpZ2h0IjoxLjY5NzgxNzY4MzIxOTkwOTd9fSx7InBhZ2VOdW1iZXIiOjExLCJiYm94Ijp7InRvcCI6NDkuNjI5NTE2NjAxNTYyNSwibGVmdCI6Ni41ODgyMzUzNzgyNjUzODEsIndpZHRoIjozMy4zOTEzNzY0OTUzNjEzMywiaGVpZ2h0IjoxLjY5NzgxNzY4MzIxOTkwOTd9fSx7InBhZ2VOdW1iZXIiOjExLCJiYm94Ijp7InRvcCI6NTEuMjk2MTg4MzU0NDkyMTksImxlZnQiOjYuNTg4MjM1Mzc4MjY1MzgxLCJ3aWR0aCI6ODcuMjMyMzIyNjkyODcxMSwiaGVpZ2h0IjoxLjY5NzgxNzY4MzIxOTkwOTd9fV19LCJjaHVua0lkIjoiYWRjZDE5NmYtYTE2ZS00YTE3LWI2YTItMDU3YTY0NWUxZWM3IiwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiMTExMiJ9fV0%3D. Additionally, the development of regulatory criteria and approval from authorities such as the World Health Organization or the US Food and Drug Administration is essential to ensure the safe and standardized application of these novel sanitizers (Kolbe et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkJqb2VybiBILiIsImZhbWlseSI6IktvbGJlIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkJyeWFuIFIuIiwiZmFtaWx5IjoiQ29hZCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLYXRoYXJpbmEiLCJmYW1pbHkiOiJSaWNodGVyIn19XSwiaWRzIjp7Imlzc24iOlsiMTMyNC00MjcyIiwiMjIwMS05MTg5Il0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDcxL21hMjQwMjQiLCJvcGVuYWxleCI6Ilc0Mzk2ODE2MzcwIiwib3BlbkFjY2Vzc0lkIjoiODg2NDA5NDkzMDYzMzY4MjE1NCJ9LCJ0aXRsZSI6IlBsYXNtYS1hY3RpdmF0ZWQgd2F0ZXLigJlzIHBvdGVudGlhbCBjb250cmlidXRpb24gdG8g4oCYT25lIEhlYWx0aOKAmSIsImFic3RyYWN0IjoiRm9vZGJvcm5lIHBhdGhvZ2VucyBjYXVzZSBhIG1ham9yIGJ1cmRlbiB0byBwdWJsaWMgaGVhbHRoIGFuZCB0aGUgZWNvbm9teSwgY29zdGluZyBBJDIuNDQgYmlsbGlvbiwgYW5kIGNhdXNpbmcgNDgsMDAwIGhvc3BpdGFsaXNhdGlvbnMgYW5udWFsbHkgaW4gQXVzdHJhbGlhLiBXaXRoIGFuIGluY3JlYXNpbmcgZ2xvYmFsIGltcGFjdCBvZiBmb29kYm9ybmUgaWxsbmVzc2VzIGFuZCB0aGUgZW1lcmdlbmNlIG9mIGFudGliaW90aWMtcmVzaXN0YW50IHBhdGhvZ2VucywgbmV3IGRlY29udGFtaW5hdGlvbiB0ZWNobm9sb2dpZXMgc2hvdWxkIGNvbnNpZGVyIHRoZSBPbmUgSGVhbHRoIGFwcHJvYWNoIHRvIGh1bWFuLCBhbmltYWwgYW5kIGVudmlyb25tZW50YWwgaGVhbHRoLiBUaGlzIHJldmlldyBleHBsb3JlcyB0aGUgYXBwbGljYXRpb24gb2YgcGxhc21hLWFjdGl2YXRlZCB3YXRlciAoUEFXKSBhcyBhIG5vdmVsIHNhbml0aXNhdGlvbiBtZXRob2QuIFdlIGRpc2N1c3MgdGhlIGltcGxpY2F0aW9ucyBvZiBhZG9wdGluZyBQQVcgYXMgYW4gZW52aXJvbm1lbnRhbGx5IGZyaWVuZGx5IGFuZCBjb3N0LWVmZmVjdGl2ZSBzYW5pdGlzZXIgdGhyb3VnaCBhIG11bHRpZGlzY2lwbGluYXJ5IE9uZSBIZWFsdGggcGVyc3BlY3RpdmUuIFRoZSBmaW5kaW5ncyB1bmRlcnNjb3JlIHRoZSBwcm9taXNpbmcgcm9sZSBvZiBQQVcgaW4gbWl0aWdhdGluZyBmb29kYm9ybmUgcGF0aG9nZW5zLCBvZmZlcmluZyBhIGhvbGlzdGljIHNvbHV0aW9uIHRoYXQgYWxpZ25zIHdpdGggdGhlIHByaW5jaXBsZXMgb2YgT25lIEhlYWx0aCBmb3IgZW5oYW5jZWQgZm9vZCBzYWZldHkgYW5kIHB1YmxpYyBoZWFsdGguIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjEwNzEvbWEyNDAyNCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cucHVibGlzaC5jc2lyby5hdS9tYS9wZGYvTUEyNDAyNCIsImlzc3VlZERhdGUiOiIyMDI0LTA1LTEwIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiTWljcm9iaW9sb2d5IEF1c3RyYWxpYSIsImlzc3VlIjoyLCJ2b2x1bWUiOjQ1LCJpbXBhY3RGYWN0b3IiOiIxLjc4In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6ODMsImxhc3RQYWdlIjo4N319LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJDU0lSTyBQdWJsaXNoaW5nIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjoyNywiY2l0ZWRCeUNvdW50IjozfX0sInF1b3RlIjoiVGhlcmVmb3JlLCBwcm90b2NvbHMgZm9yIFBBVyBhcHBsaS1jYXRpb24gc2hvdWxkIGJlIGRldmVsb3BlZCB0byBlbnN1cmUgcHJvcGVyIHVzZSBvZiB0aGlzICBub3ZlbCBzYW5pdGlzZXIuIEZ1cnRoZXJtb3JlLCByZWd1bGF0b3J5IGNyaXRlcmlhIHNob3VsZCBiZSAgZGV2ZWxvcGVkIGZvciB0aGUgc3VjY2Vzc2Z1bCBhcHBsaWNhdGlvbiBvZiBQQVcsIGFzIHRvIG91ciAga25vd2xlZGdlIFBBVyBpcyBub3QgeWV0IGFwcHJvdmVkIGJ5IGF1dGhvcml0aWVzIGxpa2UgdGhlICBXb3JsZCBIZWFsdGggT3JnYW5pemF0aW9uIG9yIFVTIEZvb2QgYW5kIERydWcgIEFkbWluaXN0cmF0aW9uLiBUaGVzZSBndWlkZWxpbmVzIHdvdWxkIGVuc3VyZSBvcHRpbWFsIHBlci1mb3JtYW5jZSBmb3IgZGlmZmVyZW50IGFwcGxpY2F0aW9ucy41IFRoZSByZW1haW5pbmcgY2hhbGxlbmdlcyBhcmUgdGhlIG5lY2Vzc2FyeSBxdWFsaXR5IGF0dHJpLSBidXRlcyBhbmQgbGltaXRhdGlvbnMgdG8gYmF0Y2ggcHJvZHVjdGlvbiBvZiBQQVcgd2l0aCB2b2x1bWVzIG9mIHVwIHRvIDUyMCBMIGjigJMxLiBBZGRyZXNzaW5nIHRoZXNlIGxpbWl0YXRpb25zICB3b3VsZCBlbmFibGUgYW4gZWZmaWNpZW50IHV0aWxpc2F0aW9uIG9mIFBBVyBhcyBhICBjb21tZXJjaWFsLXNjYWxlIHNhbml0aXNlciB3aXRoaW4gdGhlIGZvb2QgaW5kdXN0cnkuMTMsMjkgQ29uY2x1c2lvbiBUaGUgYXBwbGljYXRpb24gb2YgUEFXIGFzIGZvb2Qgc2FuaXRpc2VyLCBmZXJ0aWxpc2VyIGFuZCBkZWdyYWRhdGlvbiBhZ2VudCBmb3IgZW52aXJvbm1lbnRhbCBwb2xsdXRhbnRzIGNvdWxkIHBvc2ktdGl2ZWx5IGNvbnRyaWJ1dGUgdG8gT25lIEhlYWx0aC4gQmVzaWRlcyBmb29kYm9ybmUgcGF0aG9nZW4gIHJlZHVjdGlvbiwgUEFXIGNvdWxkIGVmZmljaWVudGx5IHJlZHVjZSB0aGUgcmlzayBvZiBBTVIgIHdpdGhpbiBmb29kIHByb2R1Y3Rpb24gZHVlIHRvIHRoZSByYXBpZCBhbmQgc3luZXJnaXN0aWMgIGFudGltaWNyb2JpYWwgcmVhY3Rpb25zLCBhZGRyZXNzaW5nIHRoZSB1bm1ldCBuZWVkIGZvciAgbm92ZWwgYW50aW1pY3JvYmlhbHMuIEhvd2V2ZXIsIGZ1cnRoZXIgcmVzZWFyY2ggaXMgcmVxdWlyZWQgIHRvIHN0dWR5IHRoZSBwb3RlbnRpYWwgaW50ZXJhY3Rpb25zIG9mIFBBVyB3aXRoIHBvbGx1dGFudHMgIGFuZCB0byBwcm92ZSBpdHMgbm9uLXRveGljaXR5IGJlZm9yZSByZWd1bGF0b3J5IGFwcHJvdmFsICBjb3VsZCBlbmFibGUgaW5kdXN0cnkgYXBwbGljYXRpb24gb2YgdGhpcyBub3ZlbCBzYW5pdGlzZXIuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjozLCJiYm94Ijp7InRvcCI6NTQuNTUzODkwMjI4MjcxNDg0LCJsZWZ0Ijo2LjE1MjQ5NjMzNzg5MDYyNSwid2lkdGgiOjQyLjMzMDc3MjM5OTkwMjM0NCwiaGVpZ2h0IjoyLjAzNDIyOTUxNjk4MzAzMjJ9fSx7InBhZ2VOdW1iZXIiOjMsImJib3giOnsidG9wIjo1NS45NzQ3ODg2NjU3NzE0ODQsImxlZnQiOjYuMTUyNDk5MTk4OTEzNTc0LCJ3aWR0aCI6NDIuNzk2OTk3MDcwMzEyNSwiaGVpZ2h0IjoyLjAzNDIyOTUxNjk4MzAzMjJ9fSx7InBhZ2VOdW1iZXIiOjMsImJib3giOnsidG9wIjo1Ny40MDIzNDc1NjQ2OTcyNjYsImxlZnQiOjYuMTUyNTAxNTgzMDk5MzY1LCJ3aWR0aCI6NDIuODAxNjI4MTEyNzkyOTcsImhlaWdodCI6Mi4wMzQyMjk1MTY5ODMwMzIyfX0seyJwYWdlTnVtYmVyIjozLCJiYm94Ijp7InRvcCI6NTguODMwMDA5NDYwNDQ5MjIsImxlZnQiOjYuMTUyNTAxNTgzMDk5MzY1LCJ3aWR0aCI6NDIuODAwODI3MDI2MzY3MTksImhlaWdodCI6Mi4wMzQyMjk1MTY5ODMwMzIyfX0seyJwYWdlTnVtYmVyIjozLCJiYm94Ijp7InRvcCI6NjAuMjUwODAxMDg2NDI1NzgsImxlZnQiOjYuMTUyNDk5MTk4OTEzNTc0LCJ3aWR0aCI6NDIuNzkzNDgzNzM0MTMwODYsImhlaWdodCI6Mi4wMzQyMjk1MTY5ODMwMzIyfX0seyJwYWdlTnVtYmVyIjozLCJiYm94Ijp7InRvcCI6NjEuNjc4NDYyOTgyMTc3NzM0LCJsZWZ0Ijo2LjE1MjQ5OTE5ODkxMzU3NCwid2lkdGgiOjQyLjc5NTQwMjUyNjg1NTQ3LCJoZWlnaHQiOjIuMDM0MjI5NTE2OTgzMDMyMn19LHsicGFnZU51bWJlciI6MywiYmJveCI6eyJ0b3AiOjYzLjEwNjAyNTY5NTgwMDc4LCJsZWZ0Ijo2LjE1MjQ5OTE5ODkxMzU3NCwid2lkdGgiOjQyLjMzMTg4MjQ3NjgwNjY0LCJoZWlnaHQiOjIuMDM0MjI5NTE2OTgzMDMyMn19LHsicGFnZU51bWJlciI6MywiYmJveCI6eyJ0b3AiOjY0LjUwNjM3MDU0NDQzMzYsImxlZnQiOjYuMTUyNDk5MTk4OTEzNTc0LCJ3aWR0aCI6MjYuMjkxMDQwNDIwNTMyMjI3LCJoZWlnaHQiOjIuMDU0NzgyODY3NDMxNjQwNn19LHsicGFnZU51bWJlciI6MywiYmJveCI6eyJ0b3AiOjY1Ljk1NDYzNTYyMDExNzE5LCJsZWZ0Ijo4LjE2MTg5NDc5ODI3ODgwOSwid2lkdGgiOjQwLjMyNDcyMjI5MDAzOTA2LCJoZWlnaHQiOjIuMDM0MjI5NTE2OTgzMDMyMn19LHsicGFnZU51bWJlciI6MywiYmJveCI6eyJ0b3AiOjY3LjM4MjE5NDUxOTA0Mjk3LCJsZWZ0Ijo2LjE1MjQ5OTE5ODkxMzU3NCwid2lkdGgiOjQyLjc5NzAzMTQwMjU4Nzg5LCJoZWlnaHQiOjYuMzEwMjU3NDM0ODQ0OTcxfX0seyJwYWdlTnVtYmVyIjozLCJiYm94Ijp7InRvcCI6NzUuOTE1OTg1MTA3NDIxODgsImxlZnQiOjYuMTUyMzczMzEzOTAzODA5LCJ3aWR0aCI6OS44ODU3NTE3MjQyNDMxNjQsImhlaWdodCI6MS42NzE5NDM3ODM3NjAwNzA4fX0seyJwYWdlTnVtYmVyIjozLCJiYm94Ijp7InRvcCI6NzguNjYxNzU4NDIyODUxNTYsImxlZnQiOjYuMTUyMzY5OTc2MDQzNzAxLCJ3aWR0aCI6NDIuODAwODQyMjg1MTU2MjUsImhlaWdodCI6MTQuODYyMzcwNDkxMDI3ODMyfX1dfSwiY2h1bmtJZCI6IjNiMjYxM2JjLTVkOWItNDAxNS05ZGRiLWM2ZWJjMTBjMGRjMyIsImxvY2F0b3IiOnsicGFnZU51bWJlciI6Ijg2In19XQ%3D%3D.

Table 5: Challenges and future perspectives of nanobubble and plasma water technologies in aquaculture

| Aspect | Challenges | Future Perspectives |
| --- | --- | --- |
| Long-term effects | Assessment of impacts on aquatic organisms and environment | Comprehensive ecological studies |
| Scalability and cost | Optimization of generation systems for commercial aquaculture | Development of efficient and affordable technologies |
| Mechanisms of action | Elucidation of molecular and cellular mechanisms | Fundamental research on biological interactions |
| Synergistic applications | Integration with other sustainable aquaculture practices | Exploration of combined approaches for enhanced benefits |
| Standardization | Development of protocols and guidelines for different settings | Establishment of best practices and regulations |

Further studies focusing on the molecular and cellular level mechanisms through which nanobubbles and PAW affect aquatic organisms is needed. Research on the synergistic effects obtained by pairing nanobubble and plasma water technologies with other sustainable aquaculture practices (e.g., probiotics, biofloc technology) should be carried out. Figure 3 highlights potential synergy pathways when nanobubble and PAW interventions are combined with complementary sustainability strategies. Moreover, it is important to have a standard protocol and guidelines in utilizing these technologies across various aquaculture settings. Ethical considerations regarding the potential environmental impact and unforeseen consequences of nanomaterials require thorough investigation and regulatory oversight (Khan et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjMwLCJzdWJ0eXBlIjp7fSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2FiYSBLaHVyc2hlZWQiLCJmYW1pbHkiOiJLaGFuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkpveWRlZXAiLCJmYW1pbHkiOiJEdXR0YSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJJc2h0aXlhcSIsImZhbWlseSI6IkFobWFkIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1vaGQgQXNocmFmIiwiZmFtaWx5IjoiUmF0aGVyIn19XSwiaWRzIjp7Imlzc24iOlsiMjU5MC0xNTc1Il0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDE2L2ouZm9jaHguMjAyNC4xMDE5NzQiLCJwbWlkIjoiMzk1ODI2MzgiLCJvcGVuYWxleCI6Ilc0NDA0MTQ4MjM0IiwiZXhhIjoiaHR0cHM6Ly9wbWMubmNiaS5ubG0ubmloLmdvdi9hcnRpY2xlcy9QTUMxMTU4NTc5Ni8ifSwidGl0bGUiOiJOYW5vdGVjaG5vbG9neSBpbiBhcXVhY3VsdHVyZTogVHJhbnNmb3JtaW5nIHRoZSBmdXR1cmUgb2YgZm9vZCBzZWN1cml0eSIsImFic3RyYWN0IjoiSW4gdGhlIGZhY2Ugb2YgZ3Jvd2luZyBnbG9iYWwgY2hhbGxlbmdlcyBpbiBmb29kIHNlY3VyaXR5IGFuZCBpbmNyZWFzaW5nIGRlbWFuZCBmb3Igc3VzdGFpbmFibGUgcHJvdGVpbiBzb3VyY2VzLCB0aGUgYXF1YWN1bHR1cmUgaW5kdXN0cnkgaXMgdW5kZXJnb2luZyBhIHRyYW5zZm9ybWF0aXZlIHNoaWZ0IHRocm91Z2ggdGhlIGludGVncmF0aW9uIG9mIG5hbm90ZWNobm9sb2d5LiBUaGlzIHJldmlldyBwYXBlciBleHBsb3JlcyB0aGUgcHJvZm91bmQgcm9sZSBvZiBuYW5vdGVjaG5vbG9neSBpbiBhcXVhY3VsdHVyZSwgYWRkcmVzc2luZyBjcml0aWNhbCBpc3N1ZXMgc3VjaCBhcyBlZmZpY2llbnQgZmVlZCB1dGlsaXphdGlvbiwgZGlzZWFzZSBtYW5hZ2VtZW50LCBhbmQgZW52aXJvbm1lbnRhbCBzdXN0YWluYWJpbGl0eS4gTmFub21hdGVyaWFscyBhcmUgdXNlZCB0byBlbmhhbmNlIG51dHJpdGlvbmFsIGNvbnRlbnQgYW5kIGRpZ2VzdGliaWxpdHkgb2YgYXF1YWZlZWQsIG9wdGltaXplIGZpc2ggZ3Jvd3RoIGFuZCBoZWFsdGgsIGFuZCBpbXByb3ZlIGRpc2Vhc2UgcHJldmVudGlvbi4gTmFub3BhcnRpY2xlLWJhc2VkIHZhY2NpbmVzIGFuZCBkcnVnIGRlbGl2ZXJ5IHN5c3RlbXMgcmVkdWNlIGFudGliaW90aWMgcmVsaWFuY2UsIHdoaWxlIG5hbm8gc2Vuc29ycyBtb25pdG9yIHdhdGVyIHF1YWxpdHkgaW4gcmVhbC10aW1lLiBGdXJ0aGVybW9yZSwgbmFub3RlY2hub2xvZ3kgaGFzIHJldm9sdXRpb25pemVkIGluZnJhc3RydWN0dXJlIGRlc2lnbiwgY29udHJpYnV0aW5nIHRvIHNtYXJ0LCBzZWxmLXJlZ3VsYXRpbmcgYXF1YWN1bHR1cmUgc3lzdGVtcy4gRGVzcGl0ZSBpdHMgdmFzdCBwb3RlbnRpYWwsIGNoYWxsZW5nZXMgc3VjaCBhcyBldGhpY2FsIGNvbnNpZGVyYXRpb25zIGFuZCBsb25nLXRlcm0gc2FmZXR5IG11c3QgYmUgYWRkcmVzc2VkLiBUaGlzIHBhcGVyIGhpZ2hsaWdodHMgbmFub3RlY2hub2xvZ3kncyB0cmFuc2Zvcm1hdGl2ZSByb2xlIGluIGFxdWFjdWx0dXJlLCB1bmRlcnNjb3JpbmcgaXRzIHBvdGVudGlhbCB0byBjb250cmlidXRlIHNpZ25pZmljYW50bHkgdG8gZ2xvYmFsIGZvb2Qgc2VjdXJpdHkgdGhyb3VnaCBlbmhhbmNlZCBwcm9kdWN0aXZpdHkgYW5kIHN1c3RhaW5hYmlsaXR5LiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2ouZm9jaHguMjAyNC4xMDE5NzQiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2ouZm9jaHguMjAyNC4xMDE5NzQiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMS0wNyIsImFjY2Vzc2VkRGF0ZSI6IjIwMjYtMDIiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZvb2QgQ2hlbWlzdHJ5IFgiLCJ2b2x1bWUiOjI0LCJpbXBhY3RGYWN0b3IiOiI3LjQ1In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTAxOTc0LCJsYXN0UGFnZSI6MTAxOTc0fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkVsc2V2aWVyIEJWIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjoyNDUsImNpdGVkQnlDb3VudCI6MjR9fSwicXVvdGUiOiJEZXNwaXRlIHRoZSBwcm9taXNpbmcgcG90ZW50aWFsIG9mIG5hbm90ZWNobm9sb2d5IGluIGFxdWFjdWx0dXJlLCBpdHMgd2lkZXNwcmVhZCBhZG9wdGlvbiBmYWNlcyBzZXZlcmFsIGNoYWxsZW5nZXMgYW5kIGNvbmNlcm5zICggS29sdXB1bGEgZXQgYWwuLCAyMDI0ICkuIEV0aGljYWwgY29uc2lkZXJhdGlvbnMsIHBhcnRpY3VsYXJseSByZWdhcmRpbmcgdGhlIHBvdGVudGlhbCBlbnZpcm9ubWVudGFsIGltcGFjdCBhbmQgdW5mb3Jlc2VlbiBjb25zZXF1ZW5jZXMgb2YgbmFub21hdGVyaWFscywgcmVxdWlyZSB0aG9yb3VnaCBpbnZlc3RpZ2F0aW9uIGFuZCByZWd1bGF0b3J5IG92ZXJzaWdodC4gTW9yZW92ZXIsIGtleSBhc3BlY3RzIHN1Y2ggYXMgbG9uZy10ZXJtIHNhZmV0eSwgY29zdC1lZmZlY3RpdmVuZXNzLCBhbmQgdGhlIHNjYWxhYmlsaXR5IG9mIG5hbm90ZWNobm9sb2d5LWRyaXZlbiBhcXVhY3VsdHVyZSBwcmFjdGljZXMgcmVtYWluIGFyZWFzIGZvciBmdXJ0aGVyIHJlc2VhcmNoIGFuZCBkaXNjdXNzaW9uLiBJbiBjb25jbHVzaW9uLCBuYW5vdGVjaG5vbG9neSBtYXJrcyBhIHNpZ25pZmljYW50IHBhcmFkaWdtIiwiZXh0ZXJuYWxDYWNoZUlkIjoiOGY3ZTVlZTgtYzMwMS0zYmZmLWExYTAtZjdmOTYzOWQzNmIzIn1d. Addressing these concerns, comprehensive toxicity studies must be conducted to ensure the safety and biocompatibility of nanotherapeutics for aquatic organisms (Elgendy et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1hbWRvdWggWS4iLCJmYW1pbHkiOiJFbGdlbmR5In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNoaW1hYSBFLiIsImZhbWlseSI6IkFsaSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBaG1lZCBBYmRhbCIsImZhbWlseSI6IkRheWVtIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlJpYWQgSC4iLCJmYW1pbHkiOiJLaGFsaWwifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTW91c3RhZmEgTS4iLCJmYW1pbHkiOiJNb3VzdGFmYSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNb2hhbWVkIiwiZmFtaWx5IjoiQWJkZWxzYWxhbSJ9fV0sImlkcyI6eyJpc3NuIjpbIjA5NjctNjEyMCIsIjE1NzMtMTQzWCJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMTAwNy9zMTA0OTktMDI0LTAxNjAzLTMiLCJvcGVuYWxleCI6Ilc0NDAwNzQwMTMwIiwib3BlbkFjY2Vzc0lkIjoiODQzMzgyOTIzNzcxMTIxODQ0OCJ9LCJ0aXRsZSI6IkFsdGVybmF0aXZlIHRoZXJhcGllcyByZWNlbnRseSBhcHBsaWVkIGluIGNvbnRyb2xsaW5nIGZhcm1lZCBmaXNoIGRpc2Vhc2VzOiBtZWNoYW5pc21zLCBjaGFsbGVuZ2VzLCBhbmQgcHJvc3BlY3RzIiwiYWJzdHJhY3QiOiJBYnN0cmFjdCBUaGUgYXF1YWN1bHR1cmUgaW5kdXN0cnkgaXMgc3VmZmVyaW5nIGZyb20gc2lnbmlmaWNhbnQgZmluYW5jaWFsIHNldGJhY2tzIGR1ZSB0byBhbiBpbmNyZWFzaW5nIGZyZXF1ZW5jeSBvZiBkaXNlYXNlIG91dGJyZWFrcywgcG9zaW5nIGEgdGhyZWF0IHRvIHRoZSBzZWN0b3LigJlzIHN1c3RhaW5hYmlsaXR5LiBWYXJpb3VzIGJhY3RlcmlhbCwgdmlyYWwsIHBhcmFzaXRpYywgYW5kIGZ1bmdhbCBwYXRob2dlbnMgaGF2ZSBsZWQgdG8gbWFzc2l2ZSBtb3J0YWxpdGllcyBpbiBmYXJtZWQgZmlzaCB3b3JsZHdpZGUuIFRocm91Z2hvdXQgdGhlIHllYXJzLCB0aGUgbWFuYWdlbWVudCBvZiBmaXNoIGRpc2Vhc2VzIGhhcyBwcmVkb21pbmFudGx5IGNlbnRlcmVkIGFyb3VuZCB0aGUgdXRpbGl6YXRpb24gb2YgY29udmVudGlvbmFsIGFudGliaW90aWNzIGFuZCBjaGVtaWNhbHMuIE5ldmVydGhlbGVzcywgdGhlaXIgaW5kaXNjcmltaW5hdGUgdXNlIGhhcyBnaXZlbiByaXNlIHRvIHNlcmlvdXMgaW1wbGljYXRpb25zLCBpbmNsdWRpbmcgYW4gaW5jcmVhc2UgaW4gcmVzaXN0YW50IHBhdGhvZ2VucywgZGlzcnVwdGlvbnMgaW4gdGhlIG1ldGFib2xpYyBwcm9jZXNzZXMgb2YgZmlzaCwgZGVncmFkYXRpb24gb2YgdGhlIGFxdWF0aWMgZW52aXJvbm1lbnQsIHRoZSBwcmVzZW5jZSBvZiBkcnVnIHJlc2lkdWVzIGluIGFxdWF0aWMgcHJvZHVjdHMsIGFuZCBhIHBvdGVudGlhbCB0aHJlYXQgdG8gaHVtYW4gaGVhbHRoLiBWYXJpb3VzIGVmZmVjdGl2ZSBiaW8tYmFzZWQgYW5kIGltbXVub3Byb3BoeWxheGlzIGFsdGVybmF0aXZlIHRoZXJhcGllcyBoYXZlIGJlZW4gZGV2ZWxvcGVkIHRvIG92ZXJjb21lIHRoZXNlIGltcGVkaW1lbnRzLiBSZWNlbnQgYWx0ZXJuYXRpdmUgdGhlcmFwZXV0aWMgYXBwcm9hY2hlcyB0byBmaXNoIGRpc2Vhc2VzIGVuY29tcGFzcyBhIHJhbmdlIG9mIHN0cmF0ZWdpZXMsIGluY2x1ZGluZyBwaHl0b3RoZXJhcGV1dGljcywgbmFub3RoZXJhcGV1dGljcywgcHJvYmlvdGljcywgcHJlYmlvdGljcywgc3luYmlvdGljcywgcGhhZ2UgdGhlcmFweSwgdmFjY2luYXRpb24sIHF1b3J1bSBxdWVuY2hpbmcsIGFudGltaWNyb2JpYWwgcGVwdGlkZXMsIGJpb3N1cmZhY3RhbnRzLCBiYWN0ZXJpb2NpbnMsIHN0ZW0gY2VsbHMsIGFuZCBkaWFnbm9zdGljLWJhc2VkIHRoZXJhcHkuIEFkdmFuY2VtZW50cyBpbiBiaW90ZWNobm9sb2d5IGhhdmUgc2lnbmlmaWNhbnRseSBlbmhhbmNlZCB0aGUgZWZmaWNhY3kgb2YgdGhlc2UgdGhlcmFwaWVzLiBIb3dldmVyLCBhZGRpdGlvbmFsIHJlc2VhcmNoIGlzIGVzc2VudGlhbCB0byByZWZpbmUgdGhlIHV0aWxpemF0aW9uIG9mIHRoZXNlIHRoZXJhcGV1dGljIGFwcHJvYWNoZXMuIENyaXRpY2FsIGNvbmNlcm5zLCBzdWNoIGFzIGVmZmljYWN5LCBjb3N0LCByaXNrcywgYXZhaWxhYmlsaXR5LCBhbmQgYWR2ZXJzZSBlZmZlY3RzIG9uIGZpc2ggYW5kIHRoZSBlY29zeXN0ZW0sIG5lZWQgdG8gYmUgYWRkcmVzc2VkIHRvIGVzdGFibGlzaCBndWlkZWxpbmVzIGZvciB0aGVpciBzdXN0YWluYWJsZSBhcHBsaWNhdGlvbiBpbiBhcXVhY3VsdHVyZS4gVGhpcyByZXZpZXcgd2lsbCBpbmNyZWFzZSBhcXVhY3VsdHVyaXN0c%2BKAmSBhd2FyZW5lc3Mgb2YgcmVjZW50IHRoZXJhcGllcyB1c2VkIGluIGZpc2ggZmFybWluZywgdGhlaXIgbWVjaGFuaXNtcywgY2hhbGxlbmdlcywgYW5kIGltcGFjdHMgd2hpbGUgcHJvbW90aW5nIHRoZSBzdXN0YWluYWJpbGl0eSBvZiBjb21tZXJjaWFsIGFxdWFjdWx0dXJlLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDA3L3MxMDQ5OS0wMjQtMDE2MDMtMyIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9saW5rLnNwcmluZ2VyLmNvbS9jb250ZW50L3BkZi8xMC4xMDA3L3MxMDQ5OS0wMjQtMDE2MDMtMy5wZGYiLCJpc3N1ZWREYXRlIjoiMjAyNC0wNy0xNyIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDgiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkFxdWFjdWx0dXJlIEludGVybmF0aW9uYWwiLCJpc3N1ZSI6Nywidm9sdW1lIjozMiwiaW1wYWN0RmFjdG9yIjoiMy4yNSJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjkwMTcsImxhc3RQYWdlIjo5MDc4fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlNwcmluZ2VyIFNjaWVuY2UrQnVzaW5lc3MgTWVkaWEifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjM1NiwiY2l0ZWRCeUNvdW50IjoyMn19LCJxdW90ZSI6Iig0KSBBZGp1c3RpbmcgZG9zYWdlcyBhbmQgYXZvaWQtaW5nIGFxdWF0aWMgZW52aXJvbm1lbnRhbCBjb25kaXRpb25zIHRoYXQgY2FuIGluZmx1ZW5jZSB0aGUgZWZmaWNhY3kgYW5kIGFic29ycHRpb24gb2YgIHBoeXRvdGhlcmFwZXV0aWNzLiBOYW5vdGhlcmFwZXV0aWNzICgxKSBFbnN1cmUgdGhlIHNhZmV0eSBhbmQgYmlvY29tcGF0aWJpbGl0eSBvZiBuYW5vdGhlcmFwZXV0aWNzIGZvciBhcXVhdGljIG9yZ2FuaXNtcyBieSBjb25kdWN0aW5nIGNvbXByZWhlbnNpdmUgdG94aWNpdHkgc3R1ZGllcy4gKDIpIE9wdGltaXplIHRoZSBzaXplIG9mIG5hbm9wYXJ0aWNsZXMgdG8gIGVuaGFuY2UgdGhlaXIgdXB0YWtlIGFuZCBhYnNvcnB0aW9uIGJ5IGZhcm1lZCBmaXNoLiAoMykgQ29uZHVjdCB0aG9yb3VnaCBlbnZpcm9ubWVudGFsICBpbXBhY3QgYXNzZXNzbWVudHMgdG8gdW5kZXJzdGFuZCB0aGUgcG90ZW50aWFsIGNvbnNlcXVlbmNlcyBvZiBuYW5vdGhlcmFwZXV0aWNzIHVzZSBvbiAgYXF1YXRpYyBlY29zeXN0ZW0uICg0KSBJbXByb3ZlIGVmZmVjdGl2ZSBhbmQgc2FmZSBkZWxpdmVyeSBzeXN0ZW1zIGZvciBuYW5vdGhlcmFwZXV0aWNzLCAgZW5zdXJpbmcgdGhhdCBuYW5vcGFydGljbGVzIHJlYWNoIHRoZSB0YXJnZXQgb3JnYW5pc21zIGluIGEgY29udHJvbGxlZCBhbmQgc3VzdGFpbmVkIG1hbi1uZXIgd2l0aG91dCBjYXVzaW5nIGhhcm0gdG8gdGhlIHN1cnJvdW5kaW5nIGVudmlyb25tZW50LiAoNSkgQWRvcHRpbmcgZ3JlZW4gc3ludGhlc2lzICBtZXRob2RzIHRvIHByb2R1Y2UgbmFub3BhcnRpY2xlcyB0byBlbmhhbmNlIHRoZWlyIHRoZXJhcGV1dGljIGVmZmljYWN5LiAoNikgUHJvdmlkaW5nIGVjby1ub21pY2FsbHkgdmlhYmxlIG5hbm90aGVyYXBldXRpY3MgZm9yIGZpc2ggZmFybWVycy4gKDcpIEVuZm9yY2UgbGVnaXNsYXRpb25zIGNvbnRyb2xsaW5nIHRoZSAgYXBwbGljYXRpb24gb2YgbmFub3RoZXJhcGV1dGljcyBpbiBhcXVhY3VsdHVyZS4gUHJvYmlvdGljcywgcHJlYmlvdGljcywgYW5kIHN5bmJpb3RpY3MgKDEpIFNlbGVjdGluZyBwcm9iaW90aWMgc3RyYWlucyB0aGF0IGFyZSB3ZWxsLWFkYXB0ZWQgdG8gdGhlIHRhcmdldCBmaXNoIHNwZWNpZXMgYW5kIGhhdmUgcHJvdmVuIGJlbmVmaWNpYWwgZWZmZWN0cyBvbiBncm93dGgsIGRpc2Vhc2UgcmVzaXN0YW5jZSwgYW5kIG92ZXJhbGwgaGVhbHRoLiAoMikgQ29uZHVjdCAgbWljcm9iaWFsIGFuYWx5c2VzIHRvIGVuc3VyZSB0aGUgZG9taW5hbmNlIG9mIGJlbmVmaWNpYWwgcHJvYmlvdGljIHN0cmFpbnMuICgzKSBTZWxlY3QgcHJvYmktb3RpYyBzdHJhaW5zIHRoYXQgZG8gbm90IGFkdmVyc2VseSBpbnRlcmFjdCB3aXRoIG90aGVyIHRyZWF0bWVudHMuICgzKSBEZXZlbG9wIG5ldyBwcm9iaW90aWMgIHN0cmFpbiBjb21iaW5hdGlvbnMgdG8gb3ZlcmNvbWUgbXV0YXRpb25zIGluIG1pY3JvYmlhbCBzdHJhaW5zLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MzksImJib3giOnsidG9wIjoyMC41MTQzMDcwMjIwOTQ3MjcsImxlZnQiOjEwLjY0NTE3MzA3MjgxNDk0MSwid2lkdGgiOjc4LjcxMTgwNzI1MDk3NjU2LCJoZWlnaHQiOjIuMTUyMDIwMjE1OTg4MTU5fX0seyJwYWdlTnVtYmVyIjozOSwiYmJveCI6eyJ0b3AiOjIyLjI0MTMzODcyOTg1ODQsImxlZnQiOjEwLjY0NTE3MzA3MjgxNDk0MSwid2lkdGgiOjc5LjI1MDM5NjcyODUxNTYyLCJoZWlnaHQiOjIuMTUyMDIwMjE1OTg4MTU5fX0seyJwYWdlTnVtYmVyIjozOSwiYmJveCI6eyJ0b3AiOjIzLjk2ODM3MDQzNzYyMjA3LCJsZWZ0IjoxMC42NDUxNzMwNzI4MTQ5NDEsIndpZHRoIjoxNi4zNzY0MDM4MDg1OTM3NSwiaGVpZ2h0IjoyLjE1MjAyMDIxNTk4ODE1OX19LHsicGFnZU51bWJlciI6MzksImJib3giOnsidG9wIjozMC4wMTAxNjYxNjgyMTI4OSwibGVmdCI6MTAuNjQ1MTc0MDI2NDg5MjU4LCJ3aWR0aCI6MTYuOTgwOTA3NDQwMTg1NTQ3LCJoZWlnaHQiOjEuNzE3MzUxOTEzNDUyMTQ4NH19LHsicGFnZU51bWJlciI6MzksImJib3giOnsidG9wIjozMy40MjEwNzM5MTM1NzQyMiwibGVmdCI6MTAuNjQ1MTc0MDI2NDg5MjU4LCJ3aWR0aCI6NzkuMjU0OTUxNDc3MDUwNzgsImhlaWdodCI6MTcuNjk1MzA4Njg1MzAyNzM0fX0seyJwYWdlTnVtYmVyIjozOSwiYmJveCI6eyJ0b3AiOjU1LjAwNDgzNzAzNjEzMjgxLCJsZWZ0IjoxMC42NDUxNzQwMjY0ODkyNTgsIndpZHRoIjozNC41NTU4MzU3MjM4NzY5NSwiaGVpZ2h0IjoxLjcxNzM1MTkxMzQ1MjE0ODR9fSx7InBhZ2VOdW1iZXIiOjM5LCJiYm94Ijp7InRvcCI6NTguNDE1NzQ4NTk2MTkxNDA2LCJsZWZ0IjoxMC42NDUxNzQwMjY0ODkyNTgsIndpZHRoIjo3OS4yNDM5MDQxMTM3Njk1MywiaGVpZ2h0IjoyLjE1MjAyMDIxNTk4ODE1OX19LHsicGFnZU51bWJlciI6MzksImJib3giOnsidG9wIjo2MC4xNDI3ODQxMTg2NTIzNDQsImxlZnQiOjEwLjY0NTE3NDAyNjQ4OTI1OCwid2lkdGgiOjc5LjI0NjI4NDQ4NDg2MzI4LCJoZWlnaHQiOjIuMTUyMDIwMjE1OTg4MTU5fX0seyJwYWdlTnVtYmVyIjozOSwiYmJveCI6eyJ0b3AiOjYxLjg2OTgxMjAxMTcxODc1LCJsZWZ0IjoxMC42NDUxNzQwMjY0ODkyNTgsIndpZHRoIjo3OC43MDc5MjM4ODkxNjAxNiwiaGVpZ2h0IjoyLjE1MjAyMDIxNTk4ODE1OX19LHsicGFnZU51bWJlciI6MzksImJib3giOnsidG9wIjo2My41OTY4NDM3MTk0ODI0MiwibGVmdCI6MTAuNjQ1MTc0MDI2NDg5MjU4LCJ3aWR0aCI6NzkuMjU2ODgxNzEzODY3MTksImhlaWdodCI6Mi4xNTIwMjAyMTU5ODgxNTl9fSx7InBhZ2VOdW1iZXIiOjM5LCJiYm94Ijp7InRvcCI6NjUuMzIzODgzMDU2NjQwNjIsImxlZnQiOjEwLjY0NTE3NDAyNjQ4OTI1OCwid2lkdGgiOjc5LjI0NDE0ODI1NDM5NDUzLCJoZWlnaHQiOjIuMTUyMDIwMjE1OTg4MTU5fX1dfSwiY2h1bmtJZCI6ImE4ODAzNDkwLTcxZGUtNDVkNS1iMmEwLWIwYjY5NDFhMjQxYiIsImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjkwNTYifX1d.

Figure 3: Potential synergistic effects of combining nanobubble and plasma water technologies with other sustainable aquaculture practices

Conclusion

Nanobubble and plasma water technologies offer promising solutions to address the challenges faced by the aquaculture industry. These technologies have demonstrated potential in improving water quality, controlling diseases, and promoting the health and growth of aquatic organisms. The unique properties and mechanisms of action of nanobubbles and PAW make them effective tools for sustainable aquaculture practices. However, further research is needed to optimize their application, assess long-term impacts, and develop cost-effective and scalable systems. As the aquaculture industry continues to grow, the integration of nanobubble and plasma water technologies holds great promise for enhancing productivity, sustainability, and eco-friendliness in fish and shellfish farming.

References

- Elgendy, M. Y., Ali, S. E., Dayem, A. A., Khalil, R. H., Moustafa, M. M., & Abdelsalam, M. (2024). Alternative therapies recently applied in controlling farmed fish diseases: mechanisms, challenges, and prospects. Aquaculture International, 32(7), 9017. https://doi.org/10.1007/s10499-024-01603-3

- Elshafy, M. B. A., El-Monem, A. I. M. A., Khattab, I. M., Abdo, S. E., Fadl, S. E., & Khadiga, G. A. (2024). Dietary feed nanozeolite, Pediococcus, and medium-chain fatty acid enhanced growth performance and transcription of growth-related gene of Nile tilapia (Oreochromis niloticus). Aquaculture International, 32(5), 5911. https://doi.org/10.1007/s10499-024-01448-w

- Hadinoto, K., Rao, N. R. H., Astorga, J. B., Zhou, R., Biazik, J., Zhang, T., Cullen, P. J., Prescott, S. W., Henderson, R. K., & Trujillo, F. J. (2022). Hybrid Plasma-Bubble Discharges for Energy-Efficient Production of Plasma-Activated Water. SSRN Electronic Journal. https://doi.org/10.2139/ssrn.4115167

- Hutagalung, S. S., Rafryanto, A. F., Sun, W., Juliasih, N., Aditia, S., Jiang, J., Arramel, A., Dipojono, H. K., Suhardi, S. H., Rochman, N. T., & Kurniadi, D. (2023). Combination of ozone-based advanced oxidation process and nanobubbles generation toward textile wastewater recovery. Frontiers in Environmental Science, 11. https://doi.org/10.3389/fenvs.2023.1154739

- Jia, M., Farid, M. U., Kharraz, J. A., Kumar, N. M., Chopra, S. S., Jang, A., Chew, Y. M. J., Khanal, S. K., Chen, G., & An, A. K. (2023). Nanobubbles in water and wastewater treatment systems: Small bubbles making big difference [Review of Nanobubbles in water and wastewater treatment systems: Small bubbles making big difference]. Water Research, 245, 120613. Elsevier BV. https://doi.org/10.1016/j.watres.2023.120613

- Khan, S. K., Dutta, J., Ahmad, I., & Rather, M. A. (2024). Nanotechnology in aquaculture: Transforming the future of food security [Review of Nanotechnology in aquaculture: Transforming the future of food security]. Food Chemistry X, 24, 101974. Elsevier BV. https://doi.org/10.1016/j.fochx.2024.101974

- Kolbe, B. H., Coad, B. R., & Richter, K. (2024). Plasma-activated water’s potential contribution to ‘One Health.’ Microbiology Australia, 45(2), 83. https://doi.org/10.1071/ma24024

- Kumar, P. (2025). Biostimulatory Effects of Plasma on Seeds and Plants. International Journal for Research in Applied Science and Engineering Technology, 13(7), 1101. https://doi.org/10.22214/ijraset.2025.73152

- Montazeri, S. M., Kalogerakis, N., & Κολλιόπουλος, Γ. (2023). Effect of chemical species and temperature on the stability of air nanobubbles. Scientific Reports, 13(1). https://doi.org/10.1038/s41598-023-43803-6

- Mozzon, M., Foligni, R., Mannozzi, C., Galdenzi, F., Laurita, R., Tappi, S., & Rosa, M. D. (2023). Effect of plasma-activated water (PAW) soaking on the lipid oxidation of sardine (Sardina pilchardus) fillets. Food Research International, 176, 113823. https://doi.org/10.1016/j.foodres.2023.113823

- Nghĩa, N. H., Nguyen, N. T., Bình, P. T., May, L. T., Huy, T. T., Giang, P. T., St‐Hilaire, S., & Van, P. T. (2022). Effect of nanobubbles (oxygen, ozone) on the Pacific white shrimp (Penaeus vannamei), Vibrio parahaemolyticus and water quality under lab conditions. Fisheries and Aquatic Sciences, 25(8), 429. https://doi.org/10.47853/fas.2022.e39

- Patkaew, S., DirekbusarakoKm, S., Hırono, I., Wuthisuthimethavee, S., Powtongsook, S., & Pooljun, C. (2024). Effect of supersaturated dissolved oxygen on growth-, survival-, and immune-related gene expression of Pacific white shrimp (Litopenaeus vannamei). Veterinary World, 50. https://doi.org/10.14202/vetworld.2024.50-58

- Sakr, M., Mohamed, M. M., Kizhisseri, M. I., & Maraqa, M. A. (2025). Characterization of air submicrobubbles for water treatment under different generation conditions. Scientific Reports, 15(1). https://doi.org/10.1038/s41598-025-21293-y

- Susanti, L., Utomo, S. W., & Takarina, N. D. (2021). Sustainability and feasibility assessments of nanobubble aeration technology in economic-socio environment of Penaeus vannamei shrimp farming. BIO Web of Conferences, 33, 5005. https://doi.org/10.1051/bioconf/20213305005

- Svetovoy, V. B. (2021). Spontaneous chemical reactions between hydrogen and oxygen in nanobubbles. Current Opinion in Colloid & Interface Science, 52, 101423. https://doi.org/10.1016/j.cocis.2021.101423

- Tekile, A. K., Kim, I., & Lee, J. (2016). Extent and persistence of dissolved oxygen enhancement using nanobubbles. Environmental Engineering Research, 21(4), 427. https://doi.org/10.4491/eer.2016.028

- Thirumdas, R., Kothakota, A., Annapure, U. S., Siliveru, K., Blundell, R., Gatt, R., & Valdramidis, V. (2018). Plasma activated water (PAW): Chemistry, physico-chemical properties, applications in food and agriculture. Trends in Food Science & Technology, 77, 21. https://doi.org/10.1016/j.tifs.2018.05.007

- Vichiansan, N., Phan-umong, P., Mekwilai, T., Kumla, J., Leksakul, K., Lumyong, S., Boonyawan, D., & Poramapijitwat, P. (2023). Appropriate Parameters for growth Reduction of microorganisms in White leg Shrimp (Litopenaeus vannamei) and Splendid Squid (Loligo formosana) by Plasma Activated Water. Research Square (Research Square). https://doi.org/10.21203/rs.3.rs-3230561/v1

- Vijayaram, S., Mahendran, K., Ringø, E., Razafindralambo, H., Kannan, S., & Sun, Y. (2024). Biogenic dietary promoters in aquaculture: nature-based solutions for enhancing growth, health, and sustainability. Annals of Animal Science. https://doi.org/10.2478/aoas-2024-0115

- Vikas, & Kumar, S. (2023). Investigation the role of Plasma-Activated Water Washing and Storage on the Shelf Life of Cestrum nocturnum L. (Night-Blooming Jasmine) Flowers. Research Square (Research Square). https://doi.org/10.21203/rs.3.rs-3789835/v1

- Wu, J., Zhang, K., Cen, C., Wu, X., Mao, R., & Zheng, Y. (2021). Role of bulk nanobubbles in removing organic pollutants in wastewater treatment [Review of Role of bulk nanobubbles in removing organic pollutants in wastewater treatment]. AMB Express, 11(1). Springer Nature. https://doi.org/10.1186/s13568-021-01254-0

- Zhang, H., Wei, J., Xv, H., Khan, I., Sun, Q., Zhao, X., Gao, J., Liu, S., & Wei, S. (2024). Bactericidal efficacy of plasma-activated water against Vibrio parahaemolyticus on Litopenaeus vannamei. Frontiers in Nutrition, 11. https://doi.org/10.3389/fnut.2024.1365282

- Zhao, W., Cui, X., Wang, Z.-Q., Yao, R., Xie, S.-H., Gao, B., Zhang, C., & Niu, J. (2022). Beneficial Changes in Growth Performance, Antioxidant Capacity, Immune Response, Hepatic Health, and Flesh Quality of Trachinotus ovatus Fed With Oedocladium carolinianum. Frontiers in Immunology, 13. https://doi.org/10.3389/fimmu.2022.940929

- Zhu, N., Li, M.-Y., & Shibata, K. (2023). The Development of a High-Concentration Oxygenated Water Generator Based on Nanobubbles and Its Application. 33, 23. https://doi.org/10.3390/engproc2023055023

- Zorzi, V., Bertini, A., Robertson, A., Berardinelli, A., Palmisano, L., & Parrino, F. (2023). The application of advanced oxidation processes including photocatalysis-based ones for the off-flavours removal (GSM and MIB) in recirculating aquaculture systems. Molecular Catalysis, 551, 113616. https://doi.org/10.1016/j.mcat.2023.113616 s