FORAMINIFERA BIOSTRATGRAPHIC AND PALEOENVIRONMENTAL ANALYSIS OF “X-1” WELL, ON THE WESTERN SECTOR OF THE NIGER DELTA BASIN, NIGERIA

ABSTRACT

A foraminiferal biostratigraphic and paleoenvironmental study was conducted on ditch-cutting samples from well X-1 (1870–1320 m), located in the western sector of the Niger Delta Basin, southern Nigeria. The objectives were to determine the stratigraphic age, depositional paleoenvironments, sequence-stratigraphic framework, and preliminary petroleum potential of the penetrated interval. A total of 48 ditch-cutting samples were analyzed using standard lithologic (visual and microscopic) and micropaleontological preparation techniques. Lithostratigraphic analysis identified four sub-lithofacies units (X1–X4) characterized by alternating sand and shale, a typical feature of the Agbada Formation. Foraminiferal analysis revealed moderate abundance and diversity of planktonic, calcareous benthic, and agglutinated benthic forms. A total of 60 foraminiferal species were identified, comprising 21 planktonic species (42%) and 39 benthic species (58%). The co-occurrence of key taxa including Globigerinoides obliquus, Globorotalia acostaensis, Globigerinoides obliquus extremus, and Cyclammina cf. minima enabled recognition of two planktonic foraminiferal zones: Globorotalia acostaensis Zone (N16) and a composite Globorotalia humerosa / G. margaritae margaritae / G. margaritae evoluta Zone (N17–N19). A composite benthic foraminiferal zone spanning Marginulina costata Zone (NNDF 04), Cyclammina minima Zone (NNDF 05), Haplophragmoides narivaensis Zone (NNDF 06), and Ammobaculites agglutinans / Florius costiferum Zone (NNDF 07) was erected, in accordance with the standard Niger Delta chronostratigraphic framework (Adegoke et al., 2017). Biostratigraphic results indicate that the studied interval penetrates sediments of Late Miocene to earliest Early Pliocene age. Paleoenvironmental interpretation, based on biofacies and lithofacies integration, identified four paleodepositional/paleobathymetric settings: coastal delta plain, inner neritic, middle neritic, and outer neritic. Sequence-stratigraphic analysis delineated a major maximum flooding surface (MFS) at 1390 m (~5.0 Ma), separating a transgressive systems tract (TST) below from a highstand systems tract (HST) above. The moderately well-sorted sands and enclosing shales exhibit good potential as petroleum reservoir, source, and cap rocks, respectively.

Keywords: Foraminiferal biostratigraphic, paleoenvironment, lithologic analysis, Age

determination, major bounding surfaces

INTRODUCTION

The Niger Delta Basin, situated in the Gulf of Guinea on the southern coast of Nigeria, is one of the world's most prolific hydrocarbon provinces. Formed as a passive-margin clastic wedge, it comprises three major diachronous lithostratigraphic units: the deep-marine Akata Formation (primarily source rocks), the paralic Agbada Formation (main reservoir interval with alternating sandstones and shales), and the continental Benin Formation (top seal and aquifer). The basin's depobelts prograde southwestward, with the coastal swamp depobelt (western sector) representing a key transitional zone between onshore and shallow offshore plays (Short and Stauble, 1967; Doust and Omatsola, 1990; Reijers, 2011). Hydrocarbon exploration in the Niger Delta has evolved from onshore and shallow-water Miocene–Eocene targets to deeper offshore frontiers, driven by Nigeria's reliance on petroleum revenues and the need for sustained discoveries (Adegoke et al., 2017).

Foraminiferal biostratigraphy, integrated with lithostratigraphy and sequence-stratigraphic analysis, remains a cornerstone for age dating, paleoenvironmental reconstruction, and reservoir prediction in deltaic settings. Numerous studies have applied these techniques across the basin (e.g., Petters, 1982, 1995; Ozumba, 1999; Ajayi and Okosun, 2014; Asadu and Anintah, 2023), yet detailed foraminiferal records from the western sector's (Figure1) coastal swamp depobelt remain limited, particularly for Late Miocene–Early Pliocene intervals of the Agbada Formation.

This study presents a foraminiferal biostratigraphic and paleoenvironmental analysis of ditch-cutting samples from well X-1 (1870–1320 m), located in the western sector of the Niger Delta Basin, southern Nigeria. The primary objectives were to: (1) establish the stratigraphic age of the penetrated interval; (2) reconstruct depositional paleoenvironments and paleobathymetry; (3) define a sequence-stratigraphic framework; and (4) evaluate preliminary petroleum system elements.

A total of 48 ditch-cutting samples were examined using standard lithologic (visual and microscopic) and micropaleontological preparation methods. Lithostratigraphic analysis delineated four sub-lithofacies units (X1–X4) dominated by sand–shale alternations, characteristic of the Agbada Formation. Foraminiferal recovery showed moderate abundance and diversity of planktonic, calcareous benthic, and agglutinated benthic forms, yielding 60 species (21 planktonic [42%], 39 benthic [58%]). Key index taxa, including Globigerinoides obliquus, Globorotalia acostaensis, Globigerinoides obliquus extremus, and Cyclammina cf. minima, supported recognition of two planktonic zones: Globorotalia acostaensis Zone (N16) and a composite Globorotalia humerosa / G. margaritae margaritae / G. margaritae evoluta Zone (N17–N19). A composite benthic zone spanning Marginulina costata Zone (NNDF 04), Cyclammina minima Zone (NNDF 05), Haplophragmoides narivaensis Zone (NNDF 06), and Ammobaculites agglutinans / Florius costiferum Zone (NNDF 07) was established, aligned with the Niger Delta chronostratigraphic scheme (Adegoke et al., 2017).

The integrated biostratigraphy dates the interval to Late Miocene to earliest Early Pliocene. Paleoenvironmental reconstruction, based on biofacies and lithofacies integration, identified four settings: coastal delta plain, inner neritic, middle neritic, and outer neritic. Sequence-stratigraphic interpretation delineated a major maximum flooding surface (MFS) at 1390 m (~5.0 Ma), separating a transgressive systems tract (TST) below from a highstand systems tract (HST) above. The moderately well-sorted sands and enclosing shales exhibit good potential as reservoir, source, and cap rocks, respectively.

This work will contributes to refining the biostratigraphic and depositional framework of the western Niger Delta's Agbada Formation, supporting ongoing exploration in the coastal swamp depobelt.

Figure 1: Location map of the Niger Delta showing “X-1 Well

(Modified from Imasuen et al., 2011).

GEOLOGICAL SETTING

The Niger Delta Basin is a world-class clastic wedge formed at the intersection of the African and South American plates, representing a classic passive-margin delta system. It exhibits characteristics of an intermediate-class delta with mixed fluvio-deltaic, wave, and tidal influences, currently in a dynamic equilibrium between accommodation space and sediment supply (Chiaghanam et al., 2011). The basin is subdivided into three major diachronous lithostratigraphic units from base to top: the Akata Formation, the Agbada Formation, and the Benin Formation (Figure 2), overlain by Quaternary deposits of varying types (Boboye and Fawora, 2007; Reijers, 2011).

According to Adegoke et al. (2017), these subsurface units consist of:

- Akata Formation: the basal, predominantly marine shale sequence, approximately 600 m thick in many areas but thickening significantly basinward. It is overpressured in deeper sections and occurs at depths of 4–6 km onshore and 1–3 km offshore (Oti and Beka, 1995). Composed mainly of marine shales with minor turbidites, it serves as the primary hydrocarbon source rock in the basin due to its high organic content.

- Agbada Formation: the most extensively studied and hydrocarbon-productive unit, forming the paralic (delta-front to pro-delta) transition between the underlying Akata and overlying Benin formations. Its top is defined by the first occurrence of shales containing brackish to marine fauna, while the base corresponds to the deepest significant sandstone with plant remains and mica, coinciding with the top of the marine Akata shales. The formation reaches thicknesses of up to ~4500 m in the central depobelt (Oti and Beka, 1995) and is characterized by rhythmic alternations of sandstone and shale that constitute the principal petroleum reservoirs. Recent geophysical studies in the southeastern Niger Delta have further highlighted structural controls and geothermal implications within this interval (Ekpo et al., 2024b&c).

- Benin Formation: the uppermost, continental to coastal-plain clastic wedge, dominated by sands and sandstones (70–100% sand content) with minor shale intercalations (Ojo and Gbadamosi, 2013). Its top coincides with the present-day surface, and the base is marked by the highest shale bearing marine foraminifera (Short and Stauble, 1967). The formation is approximately 2000 m thick (Beka and Oti, 1995) and serves primarily as an aquifer and top seal in many fields.

The outcropping equivalents of these units are the Imo Shale (Akata equivalent), Ameki Group (Agbada equivalent), and Benin Sands (Benin equivalent), respectively. The basin's structural framework is dominated by growth faults, roll-over anticlines, and shale diapirism, which play critical roles in hydrocarbon trapping and migration. In the western sector (coastal swamp depobelt), these elements combine with progradational deltaic processes to create favorable conditions for stratigraphic and structural plays, as documented in regional studies (Ekpo et al., 2024a&d; Adegoke et al., 2017).

Figure 2: Stratigraphic data sheet (eastern Niger Delta) of the Niger Delta. (from Reijers, 2011)

MATERIALS AND METHODS

Ditch-cutting samples from well X-1 (depth interval 1870–1320 m) were provided by an operating oil company in the Niger Delta. A total of 48 samples were collected at regular 10 m intervals for integrated lithologic and foraminiferal biostratigraphic analysis.

Lithologic Analysis

Lithostratigraphic description was performed using both macroscopic (visual inspection) and microscopic techniques. Each sample was examined for textural characteristics (grain size, sorting, roundness, and angularity), colour, sedimentary structures (where visible), and accessory minerals (e.g., pyrite, carbonaceous matter, ferruginous material, mica, and shell fragments). Observations were recorded systematically to delineate lithofacies units and infer depositional characteristics.

Foraminiferal Biostratigraphic Analysis

Sample preparation and analysis followed standard micropaleontological procedures for ditch cuttings (e.g., Bolli and Saunders, 1985; Petters, 1995; Adegoke et al., 2017). The workflow was as follows:

1. Sample Layout and Labelling

Samples were arranged sequentially by depth and clearly labelled.

2. Disaggregation

Unwashed ditch cuttings were rinsed with water to remove drilling mud, then air-dried. A standard 20 g subsample of each dried sample was placed in a clean sample plate, soaked in a solution of anhydrous sodium carbonate and water, and left to disaggregate overnight.

3. Washing and Sieving

Disaggregated residues were washed under running tap water over a 63 µm mesh sieve to remove clay and silt fractions. The retained residue (>63 µm) was oven-dried on a hotplate at a controlled temperature not exceeding 20° C to prevent thermal damage to foraminiferal tests.

4. Size Fractionation and Examination

Dried residues were dry-sieved into three main size fractions: coarse (>500 µm), medium (250–500 µm), and fine (63–250 µm). Each fraction was examined separately in a picking tray under reflected light using a binocular stereomicroscope (Zeiss model). Specimens were picked using a fine sable-hair brush (size N0000).

5. Identification and Documentation

Foraminiferal taxa were identified by direct comparison with published illustrations and descriptions from key references, including Bolli and Saunders (1985), Petters (1995), Adegoke et al. (2017), and other relevant regional and global monographs. Both planktonic and benthic (calcareous and agglutinated) forms were recorded. Species-level identifications were prioritized for biostratigraphic index taxa, while generic or higher-level assignments were used where preservation was poor.

All analyses were conducted at the micropaleontology laboratory of the Department of Geology, Akwa Ibom State University. No quantitative abundance data (e.g., specimens per gram) were calculated beyond total counts and qualitative abundance/diversity assessments, due to the nature of ditch-cutting samples and the focus on biostratigraphic zonation and paleoenvironmental reconstruction.

RESULT AND INTERPRETATION

Lithostratigraphy of ‟ X-1”- Well

Lithostratigraphic interpretation of well X-1 (1870–1320 m) was based exclusively on ditch-cutting samples, as no wireline logs were available. Four sub-lithofacies units (X1–X4) were delineated (Table 1), with a total analyzed thickness of 550 m.

Table 1: Depth and related lithofacies of sediments in ‟ X” –1 Well

| Depth (m) | Lithofacies | Dominant Lithology | Key Characteristics |
| --- | --- | --- | --- |
| 1870–1780 | X1 | Predominantly sand with traces of siltstone | Fine- to medium-grained, angular to subrounded, moderately sorted; ferruginous material |
| 1780–1610 | X2 | Light to gray-brownish fissile shale with minor sand/sandstone | Fissile to non-fissile, moderately hard; carbonaceous matter, pyrite |
| 1610–1420 | X3 | 100% sand with traces of siltstone | Fine- to coarse-grained, moderately to poorly sorted |
| 1420–1320 | X4 | Predominantly light to gray-brownish fissile shale monoliths with minor sand/siltstone | Fissile, moderately hard; shell fragments (gastropod), pyrite |

The dominant lithologies are sandstones and sand with subordinate shale and siltstone intercalations. Sand grains are predominantly fine-grained, occasionally medium- to coarse-grained, angular to subrounded, and moderately to poorly sorted, with ferruginous material in places. Shale intervals range from dark gray to brownish gray (occasionally light gray), are fissile to non-fissile, and moderately hard. Accessory components include pyrite, carbonaceous matter, ferruginous material, and minor shell fragments (gastropod).

The sandstone/shale ratio increases upward, producing an overall coarsening-upward succession. Alternating sand and shale is characteristic of the paralic Agbada Formation, and the upward increase in sand proportion reflects progressive progradation (seaward advancement) of the Niger Delta depositional system during the studied interval.

Foraminiferal Biostratigraphy

Foraminiferal analysis was performed on 48 ditch-cutting samples collected at 10 m intervals. Twenty-two samples were barren (1870–1800 m and 1650–1430 m), while the remaining intervals yielded moderate to low abundance and diversity of planktonic and benthic foraminifera (Figure 3 – biostratigraphic chart showing abundance and diversity trends).

A total of 223 foraminiferal specimens were recovered, representing 60 species: 21 planktonic species (42% of species; 119 specimens, 51.07% of population) and 39 benthic species (58% of species; 104 specimens, 48.93% of population). Among benthic forms, 16 were calcareous (42% of benthic species) and 13 agglutinated/arenaceous (26% of benthic species) (Figure 4a – bar chart of foraminiferal types; Figure 4b – pie chart of planktonic vs. benthic distribution).

Moderately abundant planktonic taxa included Globigerinoides trilobus, Globigerinoides obliquus extremus, Globigerinoides obliquus obliquus, Globigerinoides spp., and Globorotalia tumida. Calcareous benthic taxa occurred in moderate abundance, with key forms such as Hanzawaia mantaensis, Heterolepa pseudoungeriana, Hoeglundina elegans, Quinqueloculina lamarckiana, Uvigerina auberiana, Bulimina inflata, Oridorsalis umbonatus, and Globocassidulina subglobosa. Agglutinated benthic taxa included moderately abundant Trochammina spp., Cyclammina spp., Bathysiphon spp., and arenaceous indeterminates. Calcareous benthic forms exhibited higher diversity than agglutinated forms.

Foraminiferal Biozonation

Biozonation followed the schemes of Blow (1969, 1979), Berggren et al. (1995), Bolli and Saunders (1985), and the Niger Delta chronostratigraphic framework (Adegoke et al., 2017). Zonation was based on first appearance datums (FADs) and last appearance datums (LADs) of key planktonic taxa, supplemented by benthic markers (Tables 2 and 3; Figure 3).

Figure 3: Foraminfera biostratigraphic chart showing abundance and diversity of analyzed forms from   X-1”- well

4a

4b

Figure4:  The distribution of foraminiferaanalyzed forms from   X-1”- well as presented in:

(a)  bar chart showing foraminiferal types (b) pie chart for benthic and planktics

Table 2:Key foraminifera events used in‟ X-1” well

| S/N | EVENT | DEPTH (M) |
| --- | --- | --- |
| 1 | LAD Globigerinoides obliquus | 1390 |
| 2 | LAD Cyclammina cf. minima and Eggerella scabra | 1420 |
| 3 | FAD Globigerinoides obliquus extremus | 1720 |
| 4 | FAD Globorotalia acostaensis (~9.79 Ma) | 1780 |

Table3:Planktic foraminifera microzone of ‟ X-1” well

| Interval (m) | Microzone | Age | Depth Interval (m) |
| --- | --- | --- | --- |
| 1 | Globorotalia humerosa/G. margaritae margaritae / G. margaritae evoluta (N17–N19) | Mid Late Miocene – earliest Early Pliocene (Late Tortonian–Zanclean) | 1720–1320 |
| 2 | Globorotalia acostaensis (N16) | Late Miocene (Mid-Tortonian) | 1870–1720 |

Planktonic Zones

- Globorotalia acostaensis Zone (N16): 1870–1720 m (early Late Miocene, Tortonian). Lowest abundance/diversity; basal interval barren. Dominant planktonics: Globigerinoides bollii, G. trilobus immaturus, Globigerina bulloides. Highest calcareous benthic abundance/diversity in this zone (Lagena spp., Oridorsalis umbonatus, Uvigerina auberiana, Heterolepa pseudoungeriana). FAD G. acostaensis at 1780 m (~9.79 Ma) and presence of Globorotalia tumida confirm Late Miocene age (Berggren et al., 1995).

- Composite Globorotalia humerosa / G. margaritae margaritae / G. margaritae evoluta Zone (N17–N19): 1720–1320 m (late Late Miocene – earliest Early Pliocene). Higher abundance/diversity. Key taxa: Globigerinoides obliquus extremus (FAD 1720 m), Neogloboquadrina dutertrei, Globorotalia plesitiumida. Absence of key separators (G. margaritae primitiva, Sphaeroidinellopsis disjuncta) justifies composite zoning. LAD Cyclammina cf. minima and regional markers confirm N17–N19 equivalence (Adegoke et al., 2017). Correlates with Upper F9700 Zone.

Benthic Zones

A composite benthic zone was erected, spanning NNDF 07 to NNDF 04 (Adegoke et al., 2017):

- NNDF 07 (Ammobaculites agglutinans / Florius costiferum): Base of well (1870 m); arenaceous dominance.

- NNDF 06 (Haplophragmoides narivaensis): Arenaceous/calcareous transition; base ~7.72 Ma.

- NNDF 05 (Cyclammina minima): FDO C. cf. minima ~1450–1420 m; top regular occurrence of Valvulina flexilis, Quinqueloculina lamarckiana, Heterolepa pseudoungeriana.

- NNDF 04 (Marginulina costata): Uppermost; FDO C. minima ~1400 m; correlates with upper N18–lowermost N19.

Systematic paleontological descriptions were not undertaken, as comprehensive taxonomic treatments are available in Adegoke et al. (2017) and earlier works.

PALEOENVIRONMENTAL ANALYSIS

Paleoenvironmental interpretations integrate lithofacies, foraminiferal abundance/diversity, planktonic/benthic (P/B) ratio, calcareous/arenaceous benthic ratio, and key indicator taxa (following Culver, 1988; Murray, 2014; and Niger Delta studies).

Four paleobathymetric settings were recognised:

1. Coastal Delta / Non-Marine (Coastal Plain)

Intervals: 1870–1800 m and 1600–1450 m (barren of foraminifera).

Dominated by fine- to coarse-grained sand/sandstone with ferruginous and carbonaceous material. High-energy, proximal deltaic setting; absence of foraminifera due to destruction or dilution by terrigenous input.

2. Inner Neritic (Upper Shelf, ~0–50 m)

Intervals: ~1800–1750 m and 1650–1600 m.

Fine- to medium-grained clean sands with increasing shale. Low foraminiferal abundance/diversity; low P/B ratio; calcareous/arenaceous benthic ratio low. Indicators: Heterolepa pseudoungeriana, Quinqueloculina lamarckiana, Cribroelphidium gunteri. Proximal offshore/nearshore setting with terrigenous influence.

3. Middle Neritic (Mid-Shelf, ~50–100 m)

Interval: ~1750–1700 m.

Predominantly shale/silt with reduced sand (shale:sand ~7:3). Progressive increase in foraminiferal abundance/diversity; higher P/B and calcareous benthic ratios. Indicators: Cyclammina minima, Valvulina flexilis, Uvigerina spp., Eponides eshira. Intermediate holomarine setting with reduced continental influx.

4. Outer Neritic (Lower Shelf, ~100–200 m)

Interval: 1450–1370 m (peak near 1390 m).

Dominantly shale/mudstone (shale:sand 88–98%); pyrite and mica common. Highest foraminiferal abundance/diversity; peak P/B ratio; calcareous dominance. Indicators: Globorotalia spp., Globigerina spp., Neogloboquadrina spp., Cyclammina minima, Uvigerina auberiana, Globocassidulina subglobosa. Distal offshore/open marine setting.

The succession reflects overall retrogradation (deepening) punctuated by progradational pulses, consistent with Agbada Formation deltaic evolution.

SEQUENCE STRATIGRAPHY

Sequence-stratigraphic interpretation of well X-1 (1870–1320 m) was based on the integration of lithologic descriptions (ditch cuttings) and biostratigraphic data (foraminiferal abundance, diversity, and key bioevents). A comprehensive sequence-stratigraphic analysis, including detailed parasequence stacking patterns (progradational, retrogradational, or aggradational), was not possible due to the absence of wireline logs and seismic profiles. Consequently, only a limited framework could be established, focusing on the identification of one major bounding surface and two systems tracts.

Maximum Flooding Surface (MFS)

A single maximum flooding surface (MFS-X) was confidently identified at 1390 m (~5.0 Ma), based on the following integrated evidence (Table 4):

- Peak foraminiferal abundance and diversity (planktonic, calcareous benthic, and arenaceous forms)

- Highest occurrence of the planktonic taxon Globigerinoides spp. (lower part of N19 Zone)

- Lithologic shift to dominantly shale/mudstone with minor silt (fining-upward succession)

- Condensed section indicative of maximum landward shoreline position and deepest paleowater depth

This MFS correlates with the regionally significant Top F9700 Foraminifera Zone in the Niger Delta chronostratigraphic framework (Adegoke et al., 2017), which is widely recognized as a major Late Miocene–earliest Pliocene flooding event (~5.0–5.5 Ma).

Table 4: Delineated systems tracts and key surfaces in well X-1

| System Tract | Depth Interval (m) | Key Surface / Depth (m) | Age (Ma) | Epoch | Foraminiferal Zone |
| --- | --- | --- | --- | --- | --- |
| HST | 1390–1320 | MFS-X @ 1390 | ~5.0 | Late Miocene–Early Pliocene | Top of F9700 |
| TST | 1870–1390 | MFS-X @ 1390 | ~5.0 | Late Miocene–Early Pliocene | N16 to N17–N19 |

Systems Tracts

Due to data limitations, full characterization of lowstand systems tract (LST) was not possible. Two systems tracts were delineated:

- Transgressive Systems Tract (TST)

Interval: 1870–1390 m

This basal and thicker interval represents the older systems tract. It is characterized by a fining- and deepening-upward succession, with increasing shale content upward and evidence of marine transgressive pulses (shale–sand intercalations). Foraminiferal biofacies indicate deposition from coastal to outer neritic environments. The top of the TST is marked by a condensed section at the MFS-X (1390 m, ~5.0 Ma), which separates it from the overlying HST. The base of the TST was not penetrated, as 1870 m represents the lowest analyzed depth.

- Highstand Systems Tract (HST)

Interval: 1390–1320 m

This younger and thinner interval overlies the TST. It is dominated by shale with subordinate sand and siltstone pockets. Foraminiferal assemblages show continued co-occurrence of planktonic, calcareous benthic, and arenaceous forms, consistent with middle to outer neritic conditions. The base of the HST is the MFS-X (1390 m); the top could not be defined, as 1320 m is the upper limit of the analyzed interval.

PETROLEUM POTENTIAL

Sequence-stratigraphic analysis provides a powerful framework for predicting reservoir distribution, seal integrity, and source-rock potential in deltaic settings (Adegoke, 2012). In well X-1, the identified systems tracts and MFS offer favorable elements for a working petroleum system within the Agbada Formation.

- Reservoir Potential

Clean, fine- to medium-grained, moderately to well-sorted, subrounded to rounded sandstones within the transgressive systems tract (particularly the interval 1780–1720 m) represent the primary reservoir target. These sands are interpreted as marine-influenced delta-front or shoreface deposits with good lateral continuity potential. Under appropriate burial and diagenetic conditions, they are expected to exhibit excellent porosity and permeability, similar to proven Agbada reservoirs in the western Niger Delta.

- Source Rock Potential

Marine shales within the transgressive systems tract and the condensed section at the MFS (1390 m) provide good potential as intra-formational source rocks. The outer-neritic biofacies at and below the MFS indicate deposition under low-energy, oxygen-deficient conditions favorable for preservation of marine organic matter. Regionally, shales associated with ~5.0 Ma flooding events often contain Type II–III kerogen and contribute to hydrocarbon charge.

- Seal Potential

Thick, fissile marine shales in the upper TST and throughout the HST serve as effective top seals and intra-formational baffles. The shale-dominated interval capping the prospective TST sands (around 1720–1390 m) is particularly promising for stratigraphic trapping.

- Trap Mechanisms

Potential traps include stratigraphic pinch-outs of TST sands into sealing shales, as well as structural closures (growth faults and roll-over anticlines) typical of the coastal swamp depobelt. Correlation with offset wells would help confirm lateral continuity and trap geometry.

Overall, the penetrated interval in well X-1 demonstrates good preliminary petroleum prospectivity, with the 1780–1720 m sand package as the most attractive reservoir target and the MFS-related shales providing source and seal capacity. Quantitative evaluation (porosity, permeability, TOC, maturity) and integration with seismic and wireline data are recommended to de-risk and refine these prospects.

Fig: 5 The forms recovered in X-1 Well

CONCLUSION

This foraminiferal biostratigraphic and paleoenvironmental study of well X-1 (1870–1320 m) in the western sector (coastal swamp depobelt) of the Niger Delta Basin demonstrates that the penetrated interval belongs to the Agbada Formation. The succession consists of alternating sandstones and shales arranged in four sub-lithofacies units (X1–X4), exhibiting an overall coarsening-upward trend indicative of delta progradation.

Foraminiferal analysis recovered moderate abundance and diversity, yielding 60 species (21 planktonic and 39 benthic). Integrated planktonic and benthic biozonation established two planktonic zones — Globorotalia acostaensis Zone (N16) and composite Globorotalia humerosa / G. margaritae margaritae / G. margaritae evoluta Zone (N17–N19) — and a composite benthic zone spanning NNDF 07 to NNDF 04. These zones collectively date the studied interval to Late Miocene to earliest Early Pliocene (Tortonian–Zanclean), correlating with the Upper F9700 Foraminiferal Zone in the Niger Delta chronostratigraphic framework.

Paleoenvironmental reconstruction identified four progressive paleobathymetric settings: coastal delta plain (barren high-energy sands), inner neritic (low-diversity nearshore sands), middle neritic (increasing calcareous benthic forms), and outer neritic (peak abundance/diversity with dominant planktonics and calcareous benthics). Sequence-stratigraphic interpretation, constrained by the absence of wireline logs and seismic data, delineated a major maximum flooding surface (MFS-X) at 1390 m (~5.0 Ma), separating a thick transgressive systems tract (TST: 1870–1390 m) below from a thin highstand systems tract (HST: 1390–1320 m) above. This MFS correlates regionally with the Top F9700 flooding event.

The interval exhibits good preliminary petroleum potential: clean, moderately well-sorted TST sands (notably 1780–1720 m) offer strong reservoir quality; condensed marine shales at and below the MFS provide source and seal capacity; and stratigraphic/structural trapping mechanisms are likely in this growth-faulted depobelt setting.

REFERENCES

Ajayi E.O and Okosun, F.A. (2014).Planktic Foraminiferal Biostratigraphy of A, B, C, D Wells, Offshore Niger Delta, Nigeria.,4(6): 108-120

Asadu,A.N and   Anintah,G.O(2023): Foraminifera Biostratigraphy of Geo 92 Well, Onshore

Niger Delta, Nigeria.International Journal of Advanced Research in Science,

Engineering and Technology, 10 (5):20730-20740

Adegoke, O.S; Petters, S.W;Fayose, E.A,;Oyebamiji, A.S; Ajisaje, I.K,;Edet, J.J; Osterloff, P.I,;

Sonuga, M.S; Fajemila, O and Yussuph, W (2017): Cenozoic Foraminifera  Biostratigraphy of the Niger Delta. In: Adegoke OS, Oyebamiji AS, Edet JJ, Osterloff PL, Ulu OK (eds) Cenozoic foraminifera and Calcareous Nannofossil Biostratigraphy of the Niger Delta. Elsevier Inc., Amsterdam, pp 2–383 2.

Bé, A.W., Hemleben, C., Anderson, O.R., Spindler, M., Hacunda, J. and Tuntivate-Choy, S.

(1977): Laboratory and Field Observations of Living Planktonic Foraminifera.  Micropaleontology ,23, 155-179

Beka, F.T.and M. N. Oti, (1995).“The Distal Offshore Niger   Delta: Frontier Prospects of a Mature Petroleum Prov-ince,” In: M. N. Oti and G. Postma, Eds., Geology of Del-tas, A. A. Balkema, Rotterdamc, pp. 237-241.

Berggren, W.A., Kent, D.V., Swisher, C.C., Aubry, M., (1998): A Revised Cenozoic Geochronology and Chronostratigraphy. SEPM Special Publication No. 54, pp. 128–212.

Blow, W.H (1969): Late middle eocene to recent planktonic foraminiferal biostratigraphy. In:

Bronnimann P and Renz H.H (eds) Proceedings of the First International Conference on Planktonic Microfossils, vol. 1. E.J. Brill, Leiden, pp 199–422.

Blow W.H (1979): The Cainozoic Globigerinida, vol 3. E.J. Brill, Leiden, p 1413.

Boboye, O. A.  and Fowora, O. (2007). Biostratigraphy of Calcareous Nannossils of well XH-1, deep offshore, Niger Delta, Nigeria. Journal of Mining and Geology, 43(1 ): 175-186.

Bolli H.M, and Saunders J.B (1985): Oligocene to Holocene Low Latitude Planktic Foraminifera. In: Bolli HM, Saunders JB, Perch-Nielsen K, (eds). Plankton Stratigraphy, Cambridge University Press, Cambridge:115-262

Chiaghanam, O. I., Ozumba, B. M., Ladipo, K. J. Orajaka, J. P., Ofoma, A. E. & Chiadikobi K. (2011). The Role accommodation/sediment supply regime/basin Morphology in predicting Costal depositional style: A sequence stratigraphy framework approach for selected deep wells. Oligocene – Miocene Sediments of Costal Swamp in Niger Delta.Archives of Applied Science Research, 3 (5), 248-257. (Source: http://www.scholar carchchi library. com.archieve.html. August 14,2012)

Chukwu J.N, Okosun E.A, and Alkali Y. B (2012): Foraminiferal biostratigraphy and depositional environment of oloibiri– 1 well, Eastern Niger Delta Nigeria. J Geogr Geol 4(4):114–122

Culver,S.J.(1988): New foraminifera depth zonationof the north western Gulf of

Mxico.Palaios ,3 ,69-85

Ekpo, A.E., Bassey, N.E., George, N.J. and Udo, I.G. (2024a). Depth to basement estimation from aerogravity data over the Southeastern part of Niger Delta region of Nigeria. Researchers Journal of Science and Technology, 4(5), 44–66. DOI/URL: https://rejost.com.ng/index.php/home/article/view/135

Ekpo, A.E., Bassey, N.E., George, N.J. and Udo, I.G. (2024c). Curie Point Depth as a Key Indicator of Hydrocarbon Maturity in the Southeastern Niger Delta Basin: Insights From Aeromagnetic Data. Preprint available on ResearchSquare (full publication in 2024/2025). DOI: 10.21203/rs.3.rs-5194917/v1 URL: https://www.researchsquare.com/article/rs-5194917/v1

Ekpo, A. E., Bassey, N. E., &amp; George, N. J., (2024d). Aero-gravity data analysis for

delineating possible channels of mineralization migration through lineament - a case

study southeastern niger delta, nigeria. AKSU Annals of Sustainable Development, 2(1),

139-168. https://doi.org/10.60787/AASD-v2i1-35

Ekpo, A. E., Bassey, N. E., George, N. J., &amp; Udo, I. C.. (2024b). Unveiling the

Geothermal Energy Potential in the Southeastern Niger Delta: Insights from High-

Resolution Aeromagnetic Data. Geological Behaviour 8(1): 75-81.

https://doi.org/10.26480/gbr.01.2024.75.81

Hemleben, C., Spindler, M. and Roger Anderson, O. (1989): Modern Planktonic Foraminifera. Springer, New York, 363

Gradstein, F.M., Ogg, J.G., Smith, A.G., Ogg, G.M. (Eds.), (2012): The Geologic Time Scale

2012.Elsevier Publications, Oxford, United Kingdom, p. 1129.

Imasuen,I.O. Okiotor,M.E and Etobro , A. A. (2011). Reservoir Evaluationof Well A, Field Y, North- Eastern Niger Delta: A Case of Problematic sandstone.Advances in Applied Science Research, 2 (3):114-126.

Itam, A. E; Udoh, M. U and Inyang, D. O (2017): Foraminiferal Biostratigraphy and Paleoenvironmental study of “MR” well, Niger Delta, Southeastern Nigeria. International Basic and Applied Research Journal,3(3):1-15

Kobawila,N.D; Elenga,H,and Ngatse,L.R (2021):Miocene Foraminifera Biostratigraphy and

Interpretation North Deep-Sea Block of the Congolese Atlantic Basin.Open Journal of Geology, 11, 253-274

Murray, J. W. (2014). Ecology and palaeoecology of benthic foraminifera. Routledge

Ojo, A.O and Gbadamosi, A. O. (2013): Sequence Palyno-stratigraphical study of DEL-2 Well, Southwest of Niger Delta basin, Nigeria.Research Journal in Enginerering and Applied Science.2 (2) :86-94

Okosun, E. A., Chukwu, J. N., Ajayi, E.O. and Olatunji, O.A. (2012). Biostratigraphy, Depositional Environment and Sequence Stratigraphy of Akata Field, Eastern Niger Delta, Nigeria.International Journal of Scientific and Engineering Research, 3 (7) : 2229-5518.

Onyekuru, S. O., Ibelegbu, E. C., Iwuagwu, J. C., Essien, A. G., & Akaolisa C. (2012). Sequence Stratigraphic Analysis of XB Field, Central Swamp Depobelt, Niger Delta Basin, Southern Nigeria. International Journal of Geoscience, 3:237-257

Petters, S. W. (1979b). Some Late Tertiary foraminifera from parabe-1 well, Eastern Niger Delta. Revista Espanola de micropaleontologia, 11(1): 1190-133.

Petters, S. W. (1982). Central West African Cretaceous- Tertiary Benthic Foraminifera and Stratigraphy. Journal Paleontographical, 179, pp. 1-104.

Petters, S. W. (1984).An ancient submarine canyon in the Oligocene-Miocene of Western Niger Delta.Sedimentology Journal, 31, 805-810.

Petters, S. W. (1995). Foraminiferal Biofacies in the Nigerian Rift and Continental Margins Delta: In M. N. Oti and G. Patma (Eds). Geology of Deltas: A. A. Balkem publishers, Rotterdam.

Reijers, T. J. A. (2011). Stratigraphy and Sedimentology of the Niger Delta. Geologs, 17( 3): 133-162.

Short, K.C and Stauble, A.J (1967): Outline of the geology of Niger Delta. Bull Am Assoc Pet Geol 51:761–779