# Original Research Article

Magnitude Comparison and Sign Detection in Residue Number System

.

ABSTRACT

In this paper, a magnitude comparison scheme is proposed, and its theoretical validity is proven. The proposed scheme compares the relative magnitude of any two RNS numbers via sign detection. It first computes the modular differences of the residues of the two RNS numbers, X-Y=Z and Y-X=Z', and then computes the sum of the results, Z+Z',which is then proven to be equal to zero. The final implementation of the technique requires the reverse conversion of Z=(z1,z2, z3). Then sign detection is performed on Z and the results determine whether X>Y or Y>X. Theoretical analysis proves that the proposed scheme accurately compares the magnitude of any two RNS numbers and is efficient as it handles smaller numbers, leading to faster operations and when coupled with the moduli set used in this paper, it offers a large dynamic range.

Keywords: Residue Number System, Overflow Detection, Mixed Radix Digits (MRD’s), Mixed Radix Conversion (MRC), Magnitude Comparison

- Introduction

Residue Number System is a non-weighted and non-positional number system that uses residues to represent numbers with respect to a set of co-prime moduli. Due to it being a non-weighted and non-positional number system, the residue number system has a carry-free property with the capability of performing parallel and high-speed arithmetic operations and fault-tolerant operations. RNS is defined in terms of a set of relatively prime moduli set mii=1,k such that the gcdmi,mj=1 for i≠j,

while M=i=1kmi, is the dynamic range.

The residues of a decimal number X can be obtained as xi=Xmi, thus X can be represented in RNS as X=x1, x2, x3,…,xk, 0≤xi≤mi. This representation is unique for any integer X∈0, M-1 [1].

Despite the many advantages of the residue numbers system, there remain some challenges that limit its full implementation in general-purpose computing. One of such limitations, which is addressed in this paper, is determining the relative magnitude of two RNS numbers. Magnitude comparison in RNS is defined by determining whether one RNS number is greater than, less than or equal to another RNS number. This problem comes from the fact that simple inspection techniques based on digit-by-digit comparisons are ineffective, thus, the issue of magnitude comparison can only be addressed by operations involving all the digits of the residue number. Some of said approaches involve the full reverse conversion of the RNS number into its equivalent decimal or binary form and then comparing said numbers. This is usually done using either the Chinese Remainder Theorem (CRT) or the Mixed Radix Conversion (MRC) techniques. Other approaches involve approximate techniques that avoid the need for full reverse conversion, such as the diagonal function, Dynamic Range Partitioning (DRP), the use of a redundant modulus and parity checking. Most of said techniques, though effective, have their limitations as full reverse conversion is computationally expensive and other approximate techniques may require a specialized moduli set. Another approach, which is explored in this paper, is magnitude comparison via sign detection.

- Related Literature.

Determining the relative magnitude of two RNS numbers is one of the main challenges of the Residue Number System (RNS). Many techniques have been proposed to solve the problem of magnitude comparison, but each has limitations. Traditional techniques for magnitude comparison are mainly the Chinese Remainder Theorem (CRT) and the Mixed Radix Conversion (MRC) to convert the residue representation of numbers to a positional code[2]. However, these techniques are inefficient since CRT involves modulo M operations, M is the dynamic range of the number system, and MRC is slow and sequential. Another proposed technique for magnitude comparison is the diagonal function defined as the sum of suitable quotients, which provides the estimated number’s order of magnitude[3]. This approach computes the function using an extra modulus, which is then inserted into the set of moduli during the final implementation of the Residue system. The diagonal function was modified in [4] for RNS number comparators where its experimental results achieved reduced delay, power consumption and less hardware. A new algorithm based on the Mixed Radix Conversion II [5] was proposed to compare the magnitude of RNS numbers. This technique only uses small modulo operations and has an O(n) time complexity. Another technique for magnitude comparison was proposed in [6] which compares the magnitude of two RNS numbers without the need for binary conversion by grouping the RNS numbers based on modulo arithmetic into clusters. A low-cost cost low-power RNS comparison technique was proposed in [7] based on ordering the dynamic range into sequential partitions and finding the partitions with the matching operands. Following this, a technique based on diagonal functions was proposed in [8] compares numbers in RNS of the form {2n-1, 2n,2n+1} which eliminates the need for division on the large modulo 22n+1 replacing it with the use of a smaller modulo 2n+1 which reduces the computational complexity. Another paper proposes a fast and efficient RNS comparator and signed detector using the Dynamic Range Partitioning (DRP) technique [9] and expanding the unified system to a five-moduli set. The system could be used in the comparison of both signed and unsigned RNS numbers, and the results of the unified system prove its efficiency. An efficient technique for magnitude comparison based on two pairs of conjugate moduli was proposed in [10] which compares two unsigned RNS numbers based on parity, which indicates whether a number is even or odd. The proposed technique reduces the computational complexity of magnitude comparison by leveraging its structure. Experimental results of the VLSI circuit show a high throughput with low latency. Another work explores the use of the Chinese Remainder Theorem (CRT-III) and the Cyclic Jump technique for a six-moduli set to compare the magnitude of two RNS numbers[11]. The technique reduces the need for modulo M operations, which are computationally intensive, and compares RNS numbers based on parity and is suitable for hardware implementations. A signed magnitude comparator for the moduli set {2n-1, 2n,2n+1-1} was proposed in [12] which computes two subrange identifiers to efficiently detect the sign and compare the magnitude of RNS numbers without the need for a full reverse conversion.

- Justification.

Comparing two numbers to determine which is greater than the other is difficult in RNS due to it being a non-weighted and non-positional number system that represents numbers in their equivalent residue form with respect to a set of co-prime moduli.

Given the numbers 50 and 40, in decimal, it is quite easy to determine that 50 is greater than 40 but it becomes difficult to do so in RNS. The number 50 in RNS with respect to the moduli set {17, 16, 15} is (16,2,5)RNS171615 and the same for 40 is (6,8,10)RNS171615. In this form, it becomes difficult to compare these numbers to determine which is greater than the other.

Also, the numbers -7 and 7 are numbers that are quite easy to know which is greater, as one is positive the other is negative, but in RNS, the comparison is not so clear. In their RNS equivalent form, -7 is (10,9,8)RNS171615 and 7 is (7, 7, 7)RNS171615 which makes it difficult to clearly compare and determine which of these numbers is greater than the other. Another thing to note is that the sign completely vanishes from the number -7 when converted to its RNS equivalent.

Therefore, it is necessary to continue to explore the field of magnitude comparison in RNS to find new and efficient techniques to determine the relative magnitude of any two RNS numbers.

- Magnitude Comparison and Sign Detection.

Magnitude comparison determines the relative size of two numerical values. Consider two integers X and Y, such that X=x1,x2, x3,…, xn and Y={y1,y2, y3,…,yn} for the moduli set S={m1, m2, m3},

Proposition 1:

If X-Y>0    ⇒    X>Y, that is X-Y=Z then Z>0

Proposition 2:

If Y-X<0    ⇒   Y<X , that is Y-X=Z' then Z'<0

Proposition 3:

If  Z and Z' are equal in magnitude but different in sign, then

Z+Z'=0

  - Proposed Algorithms

4.1.1 Generalized Algorithms.

To compare the magnitude of two numbers X=x1,x2, x3,…, xn and Y={y1,y2, y3,…,yn} with respect to the moduli set S={m1, m2, m3,…,mn}.

- Accept the numbers X=x1,x2, x3,…, xn and Y={y1,y2, y3,…,yn} .

- From proposition (1), compute X-Y given X=x1,x2, x3,…, xn and Y={y1,y2, y3,…,yn}  such that,

Z=x1-y1m1,x2-y2m2,x3-y3m3,…,xn-ynmn)

that is Z=(z1,z2, z3,…,zn)

- From proposition (2), compute Y-X given X=x1,x2, x3,…, xn and Y={y1,y2, y3,…,yn} such that,

Z'=(y1-x1m1,y2-x2m2,y3-x3m3,…,yn-xnmn)

that is Z'=(z1',z2',z3',…,zn')

- From proposition (3), compute Z+Z'=0 given Z=(z1,z2, z3,…,zn) and Z'=(z1',z2',z3',…,zn') such that

Z+Z'=(z1+z1'm1,z2+z2'm2,z3+z3'm3,…,zn+zn'mn)

=(0, 0, 0,…,0)

- Compute  M2-1 and M-1 such that M=i=1nmi=m1∙m2∙m3…∙mn

- Perform reverse conversion on Z using the Mixed Radix Conversion (MRC) technique Were,

X= a1+a2m1+a3m1m2+…+aim1m2…mk-1			        (1)

and a1, i=1,k

the mixed radix digits (MRDs) can be computed by:

a1= x1,

a2=x2-a1∙m1-1m2m2,

a3= ((x3-a1)∙m1-1m3-a2)∙m2-1m3m3

⋮

an= ((…(xn-a1)∙m1-1mn-a2)∙m2-1mn-…-an-1)mn-1-1mnmn        (2)

- Determine whether Z falls within the range [0,  M2-1] or [M2,  M-1].

- If Z falls within the range [0,  M2-1], then Z is positive and X>Y. But if Z falls within the range [M2,  M-1], then Z is negative and Y>X.

4.1.2 Algorithms for The Moduli Set S={22n-1, 22n, 22n+1}.

To compare the magnitude of two numbers X=x1,x2, x3 and Y={y1,y2, y3} with respect to the moduli set S={22n-1, 22n, 22n+1}.

- Accept the numbers (x1,x2, x3) and (y1,y2, y3).

- From proposition (1), compute X-Y given (x1,x2, x3) and (y1,y2, y3) such that,

Z=(x1-y122n+1,x2-y222n,x3-y322n-1)				(3)

that is Z=(z1,z2, z3)

- From proposition (2), compute Y-X given (x1,x2, x3) and (y1,y2, y3) such that,

Z'=(y1-x122n+1,y2-x222n,y3-x322n-1)				(4)

that is Z'=(z1',z2',z3')

- From proposition (3), compute Z+Z'=0 given Z=(z1,z2, z3) and Z'=(z1',z2',z3') such that

Z+Z'=(z1-z1'22n+1,z2-z2'22n,z3-z3'22n-1)		(5)

=(0, 0, 0)

- Compute  M2-1 and M-1

such that M=i=1nmi=(22n+1)∙(22n)∙(22n-1)

M=26n-22n			(6)

Then,

M2-1=(26n-22n)2-1		(7)

And

M-1=26n-22n-1		(8)

- Perform reverse conversion on Z using the Mixed Radix Conversion (MRC) technique Were,

X= a1+a2(22n+1)+a3(22n+1)(22n)

X= a1+a2(22n+1)+a3(24n+22n)       (9)

the mixed radix digits (MRDs) can be computed by:

a1= z1,

a2=z2-a1∙m1-122n22n,

a3= ((z3-a1)∙m1-122n-1-a2)∙m2-122n-122n-1			(10)

- Determine whether Z falls within the range [0,  (26n-22n)2-1] or [(26n-22n)2,  26n-22n-1].

- If Z falls within the range [0,  (26n-22n)2-1], then Z is positive and X>Y. But if Z falls within the range [(26n-22n)2,  26n-22n-1], then Z is negative and Y>X.

  - Implementation.

4.2.1 Forward Conversion.

Following equations (2) and (3), the main aim in this case is to compute the residues Z=(z1,z2, z3) and Z'=(z1',z2',z3') which are to be fed into the proposed scheme for post processing.

x1       x2        x3

z1           z2         z3

Figure 1 Block Diagram Showing The Forward Conversion of The Rsidue Numbers.

4.2.1 Reverse Conversion.

In this section, we perform reverse conversion on Z using the MRC technique. From equation (8) we compute the mixed radix digits. Given the moduli set {22n-1, 22n, 22n+1} where m1=22n+1  , m2=22n  and m3=22n-1 for every integer n>1, the following hold true;

m1-1m2=1				(10)

m2-1m3=1				(11)

m1-1m3=1				(12)

Proof: it can be proven that (22n+1)(1)22n=1, then 1 is the multiplicative inverse of m1 with respect to m2. This implies

22n+1122n=22n22n+122n22n

=0+122n=122n=1

Thus, equation (10) holds true.

Similarly, if it can be proven that (22n)(1)22n=1, then 1 is the multiplicative inverse of m2 with respect to m3. This implies

22n122n-1=22n-1+1122n-1

=22n-122n-1+122n-1∙122n-1

=(0+1)∙122n-1

=(1)(1)22n-1=122n-1=1

Thus, equation (11) holds true.

In a similar way, if it can be proven that (22n+1)(1)22n-1=1 then 1 is the multiplicative inverse of m1 with respect to m3. This implies

(22n+1)(1)22n-1=22n22n-1+122n-122n-1=122n-1=1

Thus, equation (12) holds true.

Figure 2 Reverse Coversion Block Diagram of MRC implementation

- Numerical Illustrations.

This sub-section presents numerical illustrations of the proposed scheme. Using the moduli set {22n-1, 22n, 22n+1}, Let n=2 in the moduli set, then the moduli set  S={17, 16, 15}.

Illustration 1: Determining The Relative Magnitude of Two RNS numbers X=(16,2,5)RNS171615 and Y=(6,8,10)RNS171615 using the proposed algorithm.

From The Equations (3) and (4)

Compute for  Z=(z1,z2, z3) and Z'=(z1',z2',z3')

Such that,

Z=(x1-y1m1,x2-y2m2,x3-y3m3)RNS171615

x1=16,  x2=2,  x3=5

y1=6,  y2=8,  y3=10

Z=(16-617,2-816,5-1015)

Z=(1017,-616,-515)

Z=(10, 10, 10)RNS171615

Then,

Z'=(y1-x1m1,y2-x2m2,y3-x3m3)RNSm1m2m3

Z'=(6-1617,8-216,10-515)

Z'=(-1017,616,515)

Z'=(7, 6, 5)RNS171615

From Equation (5), Compute Z+Z'

Z+Z'=(z1-z1'm1,z2-z2'm2,z3-z3'm3)RNSm1m2m3

Z+Z'=(10+717,10+616,10+515)

Z+Z'=(1717,1616,1515)

Z+Z'=(0, 0, 0)RNS171615

Meaning proposition (3) holds true.

From the Equations (6), (7) and (8) Compute M and the Ranges [0,  M2-1] and [M2,  M-1].

Computing  M2-1 and M-1

From equation (6), we compute M such that,

M=212-24

M=4096-16

M=4080

Then we compute [0,  M2-1] and [M2,  M-1].

From equation (7),

M2-1=212-242-1

=4096-162-1=2039

Therefore, the first range is [0,  2039]

From equation (8),

M-1=212-24-1

=4096-16-1=4079

Also,

M2=4096-162

=40802=2040

Therefore, the second range is the [2040,  4079].

Perform Reverse Conversion Using MRC.

Find the modular inverse m1-1m2, m1-1m3, m2-1m3, such that

m1∙m1-1m2=1

m1∙m3-1m3=1

m2∙m2-1m3=1

We get

m1-1m2=1

m1-1m3=1

m2-1m3=1

From equation (10),

we compute the mixed radix digits (MRDs) such that,

a1= z1=10,

a2=10-10∙116

a2=0

a3= ((10-10)∙(1)-(0))∙(1)15

a3=0

From equation (9), compute Z such that,

Z=a1+a2m1+a3m1m2

Z=10+0(17)+0(17)(16)

Z=10

Determine if Z Falls Within the Positive Range [0, 2039] or the Negative Range [2040, 4079].

Since Z=10,

We determine that Z falls within the range 0≤Z≤ 2039. Therefore, Z is positive which implies proposition 1 holds true. This implies that X is of a greater magnitude than Y, hence X>Y.

Illustration 2: Determining The Relative Magnitude of Two RNS numbers X=(10,9,8)RNS171615 and Y=(7,7,7)RNS171615 using the proposed algorithm.

From The Equations (3) and (4)

Compute Z=(z1,z2, z3) and Z'=(z1',z2',z3')

From equation (3)

Z=(x1-y1m1,x2-y2m2,x3-y3m3)RNS171615

x1=10,  x2=9,  x3=8

y1=7,  y2=7,  y3=7

Z=(10-717,9-716,8-715)

Z=(317,216,115)

Z=(3, 2, 1)RNS171615

From equation (4)

Z'=(y1-x1m1,y2-x2m2,y3-x3m3)RNSm1m2m3

Z'=(7-1017,7-916,7-815)

Z'=(1417,1416,1415)

Z'=(14, 14,14)RNS171615

From Equation (5), Compute Z+Z'

Z+Z'=(z1-z1'm1,z2-z2'm2,z3-z3'm3)RNSm1m2m3

Z+Z'=(3+1417,2+1416,1+1415)

Z+Z'=(1717,1616,1515)

Z+Z'=(0, 0, 0)RNS171615

This implies that proposition (3) holds true.

From the Equations (6), (7) and (8) Compute M and the Ranges [0,  M2-1] and [M2,  M-1].

Computing  M2-1 and M-1

From equation (6), we compute M such that,

M=212-24

M=4096-16

M=4080

Then we compute [0,  M2-1] and [M2,  M-1].

From equation (7),

M2-1=212-242-1

=4096-162-1=2039

Therefore, the first range is [0,  2039]

From equation (8),

M-1=212-24-1

=4096-16-1=4079

Also,

M2=4096-162

=40802=2040

Therefore, the second range is the [2040,  4079].

Perform Reverse Conversion Using MRC.

Find the modular inverse m1-1m2, m1-1m3, m2-1m3, such that

m1∙m1-1m2=1

m1∙m3-1m3=1

m2∙m2-1m3=1

We get

m1-1m2=1

m1-1m3=1

m2-1m3=1

From equation (10),

we compute the mixed radix digits (MRDs) such that,

a1= z1=3,

a2=2-3∙116

a2=15

a3= ((1-3)∙(1)-15)∙(1)15

a3=8

From equation (9), compute Z such that,

Z=a1+a2m1+a3m1m2

Z=3+15(17)+8(17)(16)

Z=3+255+2176

Z=2434

Determine if Z Falls Within the Positive Range [0, 2039] or the Negative Range [2040, 4079].

Since Z=2434,

We determine that Z falls within the range 2040≤Z≤ 4079. Therefore, Z is negative which implies proposition 1 does not hold true in this case. This implies that Y is of a greater magnitude than X, hence Y>X.

- Performance Evaluation.

To evaluate the performance of the proposed scheme, it is compared with similar state of the art scheme [9].

Table 1: Area and Delay of the Proposed Scheme and [9]

| Scheme | Proposed Scheme (2) | [9] |
| --- | --- | --- |
| Area (∆FA) | 22n | 35n2+175n |
| DelayDF | 16n+3 | 14n+8log2(n)+16 |
| Area-Delay Product AD2 | 5632n3+2112n2+198n | 5n(n+5)(196n2+224nlogn+448n+64logn2+256logn+256 |

From Table 1, the comparative evaluation of the proposed scheme and [9]. This is based on three key matrices: area, delay and area-delay product for increasing values of n.

From the table, it is seen that the area of the proposed magnitude comparison scheme is given as 22n which is comparatively smaller as compared to the area of [9] which is given as, 35n2+175n. This is also seen in the delay where the proposed magnitude comparison scheme has a delay of 16n+3, which is again lower compared to 14n+8log2(n)+16 by [9]. The disparity between both metric evaluations is reflected in the area-delay product, demonstrating efficiency of the proposed scheme.

Figure 3: Graph of Area Comparison of The Proposed Scheme with Another Scheme

Figure 3 shows the comparative area evaluation of the proposed scheme and [9]. From Figure 3, we notice that the proposed scheme is smaller and increases linearly as compared to the Area in [9] which is significantly larger and increases exponentially across all values of n. This indicates that the proposed scheme is more hardware-efficient, as it requires fewer logic units and gates for implementation. From Figure 3, we notice that for increasing values of n, the area cost is minimal as compared to [9] suggesting that the proposed scheme is the more efficient and scalable scheme. Therefore, the graph effectively demonstrates that the proposed scheme is a more efficient solution compared to [9].

Figure 4: Graph of Delay Comparison of The Proposed Scheme with Another Scheme

Figure 4 shows the comparative delay evaluation for both the proposed scheme and [9]. From figure 4, the proposed scheme is shown to consistently have a lower delay than that of [9] across all values of n. For instance, at n = 4, the proposed scheme notes an approximate delay of 67 ns, while [9] exhibits a delay of about 88 ns. The lower delay values prove that the proposed scheme has a faster computation.

Figure 5: Graph of AD2 Comparison of The Proposed Scheme with [9]

Figure 5 shows the area-delay product, a combined metric of the hardware resource utilization and computational delay, capturing the tradeoffs of both. The graph shows that the proposed scheme is lower across increasing values of n, implying lower tradeoffs as compared to [9]. This proves that the proposed scheme performs better across the boards.

The scheme boasts a larger dynamic range of 4080 as compared with the dynamic range of 1680 in [9]. This implies hat the proposed scheme covers a large range of numbers in its operation as compared to [9].

Although other schemes based on Dynamic Range Partitioning (DRP) and diagonal functions are faster for specific moduli sets, the proposed scheme remains simple and general-purpose and can efficiently and accurately compare the magnitude of both signed and unsigned numbers. It can be implemented on any valid co-prime moduli set, making it scalable. Due to its simplicity, the scheme is well-suited for hardware implementation.

- Conclusion

In this paper, we proposed an efficient scheme for comparing the magnitude of any two RNS numbers. The scheme was implemented on the moduli set {22n-1, 22n, 22n+1} and numerical illustrations were carried out on the proposed scheme to show its correctness. Further evaluation proves that the scheme is more efficient as compared with other state-of-the-art schemes and precisely compares the magnitude of both signed and unsigned numbers.

# References

[1]	H. Siewobr and K. A. Gbolagade, “RNS Overflow Detection by Operands Examination,” 2014.

[2]	I. Flores, “Residue Arithmetic and Its Application to Computer Technology (Nicholas S. Szabo and Richard I. Tanaka),” SIAM Review, vol. 11, no. 1, pp. 103–104, Jan. 1969, doi: 10.1137/1011027.

[3]	G. Dimauro, S. Impedovo, and G. Pirlo, “A new technique for fast number comparison in the residue number system,” IEEE Transactions on Computers, vol. 42, no. 5, pp. 608–612, May 1993, doi: 10.1109/12.223680.

[4]	M. Babenko et al., “Rns number comparator based on a modified diagonal function,” Electronics (Switzerland), vol. 9, no. 11, pp. 1–16, Nov. 2020, doi: 10.3390/electronics9111784.

[5]	K. Isupov, “An Algorithm for Magnitude Comparison in RNS based on Mixed-Radix Conversion II,” Int J Comput Appl, vol. 141, no. 5, pp. 1–4, May 2016, doi: 10.5120/ijca2016909626.

[6]	P. Ghayour, “A New Algorithm to Compare the Magnitude of Two RNS Numbers,” Dec. 2016, [Online]. Available: http://arxiv.org/abs/1612.09168

[7]	Z. Torabi and G. Jaberipur, “Low-Power/Cost RNS Comparison via Partitioning the Dynamic Range,” IEEE Trans Very Large Scale Integr VLSI Syst, vol. 24, no. 5, pp. 1849–1857, May 2016, doi: 10.1109/TVLSI.2015.2484618.

[8]	N. Chervyakov, A. Nazarov, M. Babenko, A. Garianina, and A. Tchernykh, “The fast algorithm for number comparing in three-modular RNS,” in Proceedings - 2016 International Conference on Engineering and Telecommunication, EnT 2016, Institute of Electrical and Electronics Engineers Inc., Jan. 2017, pp. 26–28. doi: 10.1109/EnT.2016.12.

[9]	Z. Torabi and A. Belghadr, “Fast and Power Efficient Signed/Unsigned RNS Comparator & Sign Detector,” Journal of Electrical and Computer Engineering Innovations, vol. 11, no. 1, pp. 41–50, 2023, doi: 10.22061/JECEI.2022.8321.505.

[10]	L. Sousa, “Efficient Method for Magnitude Comparison in RNS Based on Two Pairs of Conjugate Moduli.”

[11]	A. Abdul-Hamid, Efficient Method For Magnitude Comparison In Rns Using A Six Moduli-Set And Based On The New Crt Iii And Hybrid Cyclic Jump Method. 2021. doi: 10.13140/RG.2.2.26318.79686.

[12]	S. Kumar and C.-H. Chang, “A VLSI-efficient signed magnitude comparator for { 2n-1, 2n, 2n +2n+1-1} RNS,” in 2016 IEEE International Symposium on Circuits and Systems (ISCAS), IEEE, May 2016, pp. 1966–1969. doi: 10.1109/ISCAS.2016.7538960.

APPENDIX

Table 2: Area and Delay Comparison of the Proposed Scheme and [9]

| n | Proposed Scheme (2) |  |  | [9] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Area | Delay | AD2 | Area | Delay | AD2 |
| 1 | 22 | 19 | 7942 | 210 | 30 | 189000 |
| 2 | 44 | 35 | 53900 | 490 | 52 | 1324960 |
| 3 | 66 | 51 | 171666 | 840 | 70.67970001 | 4196320.794 |
| 4 | 88 | 67 | 395032 | 1260 | 88 | 9757440 |
| 5 | 110 | 83 | 757790 | 1750 | 104.5754248 | 19138034.06 |
| 6 | 132 | 99 | 1293732 | 2310 | 120.6797 | 33641892.88 |
| 7 | 154 | 115 | 2036650 | 2940 | 136.4588394 | 54745783.64 |
| 8 | 176 | 131 | 3020336 | 3640 | 152 | 84098560 |
| 9 | 198 | 147 | 4278582 | 4410 | 167.3594 | 123520434.3 |
| 10 | 220 | 163 | 5845180 | 5250 | 182.5754248 | 175002375.1 |