Original Research Article

AUTOREGRESSIVE FUZZY MODELING OF POWERED ELECTRICAL DISTRIBUTIONS AND ROBUST ESTIMATION

1. Abstract

In this article, we analyzed the electrical operating voltage of a substation in Kinshasa by transforming its classical distribution into a fuzzy distribution. This approach allowed us to estimate fuzzy stochastic parameters, which we classified as total and partial parameters. Total parameters, such as fuzzy expected value and fuzzy variance, have a maximum degree of membership equal to 1. Partial parameters, such as fuzzy autocovariance and fuzzy autocorrelation, have degrees of membership less than 1. This study formalized a fuzzy distribution approach based on Zadeh arithmetic , providing a rigorous framework for modeling imprecision. The integration of fuzzy numbers into estimation methods led to a more robust evaluation of the model parameters. Furthermore, the stationarity criteria were revisited in this fuzzy context, highlighting their theoretical consistency and practical application. The results obtained confirm the value of this approach for the analysis of random phenomena tainted by epistemic uncertainty.

2. Keywords: Fuzzy Autoregressive Process ( AR), Mathematical Expectation ( Ex(et)), variance ( γo), autocorrelation ( φ), autocovariance ( γi).

3. Introduction

More recently, Benimi , CM et al. published article "A fuzzy approach to parameter estimation in autoregressive This article, titled " Models ," deals with fuzzy parameter estimation in autoregressive models, integrating concepts from fuzzy set theory and fuzzy statistics applied to AR processes. Focusing on modeling the operating voltage of an electrical power distribution substation in Kinshasa, the capital of the Democratic Republic of Congo, it proposes an original approach combining probability theory and fuzzy statistics through the fuzzification of time-domain operating voltage data. This methodology allows for the construction of a statistical distribution that incorporates both randomness and measurement imprecision. It leads to the development of a stationary fuzzy stochastic process model suitable for analyzing the electrical grid.

## N substation

UPN substation is powered by a 30 kV power line , with a voltage of setpoint of 30.5 kV and a maximum overvoltage of 32 kV.

28kV 30.5kV 32kV

The frequency remains constant at 50 Hz , and the operation reflects the daily consumption of subscribers, which has a periodic character . Given the significant voltage variations, we chose to create a fuzzy distribution based on the substation's operating voltage . The data used corresponds to 24 observations , representing the average monthly voltage over two years of operation.

| ## time | ## Kv | ## time | ## Kv | ## time | ## Kv | ## time | ## Kv |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ## January 2023 | ## 30.5 | ## July 2023 | ## 32 | ## January 2024 | ## 28 | ## July 2024 | ## 30.5 |
| ## Feb 2023 | ## 29.5 | ## August 2023 | ## 29.5 | ## Feb 2024 | ## 29.5 | ## August 2024 | ## 29.5 |
| ## March 2023 | ## 28 | ## September 2023 | ## 30.5 | ## March 2024 | ## 29.5 | ## September 2024 | ## 32 |
| ## April 2023 | ## 29.5 | ## October 2023 | ## 29.5 | ## April 2024 | ## 32 | ## October 2024 | ## 30.5 |
| ## May 2023 | ## 30.5 | ## November 2023 | ## 30.5 | ## May 2024 | ## 30.5 | ## November 2024 | ## 30.5 |
| ## June 2023 | ## 30.5 | ## Dec 2023 | ## 30.5 | ## June 2024 | ## 29.5 | ## Dec 2024 | ## 32 |

Table 1. Average monthly operating voltage of the UPN substation

5. Fuzzification of a statistical series.

The elements of the statistical series are arranged in Table 1. The subsets of Eare assigned the modality xiof the element ejfor the maximum frequency of association of pairs xi,ej, ∀i, i=1,2,…,n. In other words;

- . xej=xi⟺fi,j=maxifij.

This procedure results in the definition of a partition Eaccording to the characteristics of the character X. To make it appear more rigorously, the notations... μxejis the membership function of the character ejto the nth modality of the character, and we define it Fi,jwith respect to each modality. This implies the following conditions;

- .Fi,j= fi,jmaxifi,j   which implies

- .0≤Fi,j≤1      with in particular

- .Fi,j=1⟺fi,j=maxifi,j

- .Fi,j=0⟺fi,j=0

Hence, Fi,jit is interpreted as the degree of belonging of ejto the iiémèmodality of the statistical characteristic X, with Ethe set of statistical units ejsuch that:

E=ejj=1,2,…,n

And Xthe set of modalities xiof a statistical characteristic x;

X=xii=1,2,…,m

E is the set formed from the semi-annual average voltage measurements of the UPN substation. However, our substation is two years old, which explains the 24 observations grouped semi-annually. ej This represents the fourth semester of voltage observations that make up the set E ; E=ejj=1,2,3,4 And X represents the set of modalities which are the different operating tensions observed monthly . X=28Kv, 29,5Kv, 30,5Kv, 32Kv

Thus, we have the following table

| ## x | ## e1 | ## e2 | ## e3 | ## e4 |
| --- | --- | --- | --- | --- |
| ## 28KV | ## 1 | ## 0 | ## 1 | ## 0 |
| ## 29.5KV | ## 2 | ## 2 | ## 3 | ## 1 |
| ## 30.5KV | ## 3 | ## 3 | ## 1 | ## 3 |
| ## 32KV | ## 0 | ## 1 | ## 1 | ## 2 |

## Table 2. Different six-monthly operating voltages

## given ejsix-monthly voltage , fi,jdenotes the association frequency of the pair; xi,ejin this case, it is the proposal of the six-monthly operating voltage pair and the six-monthly workforce.

| X | ## e1 | ## e2 | ## e3 | ## e4 |
| --- | --- | --- | --- | --- |
| ## 28KV | ## 16 | ## 0 | ## 16 | ## 0 |
| ## 29.5KV | ## 26 | ## 26 | ## 36 | ## 16 |
| ## 30.5KV | ## 36 | ## 36 | ## 16 | ## 36 |
| ## 32KV | ## 0 | ## 16 | ## 16 | ## 26 |

Table 3. Frequency of couple associationxi,ej

This gives us the following table

| ## X | ## e1 | ## e2 | ## e3 | ## e4 |
| --- | --- | --- | --- | --- |
| ## 28KV | ## 0.166 | ## 0 | ## 0.166 | ## 0 |
| ## 29.5KV | ## 0.333 | ## 0.333 | ## 0.5 | ## 0.166 |
| ## 30.5KV | ## 0.5 | ## 0.5 | ## 0.166 | ## 0.5 |
| ## 32KV | ## 0 | ## 0.166 | ## 0.166 | ## 0.333 |

Table 4. Frequency of couple associationxi,ej

Starting from Fi,jwhich is interpreted as the degree of belonging ejto the i -th modality of the statistical characteristic X, starting from expression (2) of the Presentation of our statistical series :

Fi,j= fi,jmaxifi,j

Thus, the previous table becomes ;

| ## x | ## μxe1xi | ## μxe2xi | ## μxe3xi | ## μxe4xi |
| --- | --- | --- | --- | --- |
| ## 28KV | ## 0.332 | ## 0 | ## 0.332 | ## 0 |
| ## 29.5KV | ## 0.666 | ## 0.666 | ## 1 | ## 0.332 |
| ## 30.5KV | ## 1 | ## 1 | ## 0.332 | ## 1 |
| ## 32KV | ## 0 | ## 0.332 | ## 0.332 | ## 0.666 |

Table 5. Degree of membership of ejthe modalityxi

The degree to which statistical units belong to the categories of a characteristic is used to obtain a statistical table in the common sense of the term. A statistical characteristic X is therefore defined as any descriptive fact allowing a partition to be performed on a reference set E. We verify that ... i=1mfi=1gives a first idea of a fuzzy (quantitative and discrete) characteristic denoted by x... The value xejof any statistical unit does not necessarily correspond to a unique category xiof X. Moreover, the correspondence is generally imprecise. By definitionμxejxi represents the truth value of the propositionxej is worth xi. In what follows, we will choose to estimate the values of μxejxiby that of Fi,j. Consequently, the statistical series relating to the fuzzy nature is represented by Table 5 , in which the statistical units are characterized "more or less" by the modalities of x. It should be noted that the value μxejxi=1 is a special case imposed by the number theory that we examine in the following paragraph.

μxejxi=1 , "does not always mean thatxej equals exactly 1, but that according to the definition of Fi,j,xi is the most credible value of xej. In any case, it is possible to assimilatexej to a vague number whose definition is recalled below.

We have a fuzzy number taking its values from a discrete set X formed by the operation of a xejSnel /UPN substation. This is a fuzzy subset of X such that:

xej=xi,μxejxi,∀xi∈X

We will agree to call the characteristic diagram of the fuzzy number the graphical representation of the values of the membership function in the most general case. Then xejit represents the measurement of a quantity within its interval of uncertainty.

μx(e1)

1

0.66

0.33

0 28 29.5 30.5 32e1

Fig. 1. Representation of e1and μx(e1)case of Snel / UPN

In this case, μxejxi=1it means that xejis exactly one of the values of a,c. By definition, we have ;

μxejxi=1 ; ∀xi∈a,cμxejxi=0 ; ∀xi∉a,c

## The membership function takes the value 1 for the most credible value of the fuzzy number; we can then understand the reason why the membership functions are currently estimated by the ratio .

## Fi,j= fi,jmaxifi,j

μR

1 1

0.9 0.9

0.8 0.8

0.7 0.7

0.6 0.6

0.5 0.5

0.4 0.4

0.3 0.3

0.2 0.2

0.1 0.1

0e1               e2           e3e 4 e

x1

x2

x3

x4

x μxe1(x1)    μxe2(x2)     μxe3(x3)     μxe4(x4)

Fig. 2. Representation of μxej, ej et xiSnel /UPN case

## 6. Fuzzy Mathematical Expectation

## Fuzzy expectation is, in other words, an arithmetic mean of xej, denoted Exej . It is expressed by the expression ;

Exej =1n ∑j=1nx(ej) ∑ denotes the "fuzzy sum" operator

The addition ⊕of two fuzzy numbers xejand xej'is the fuzzy subsetxej ⊕ xej'such as :

xej ⊕ xej' =xej +xej' ,μxej ⊕ xej'

zadet 's extension theory , we can write:

μxej⊕ μxej'=maxyminj,j'μxej,μxej'

With y=xej +xej'

The different y-values found are related to the different paths found in our trees. Each tree has 64 paths, and in our trees, there are some paths that are similar in terms of y-values. We will use the part tree to make it easier to accurately calculate the expected value.

μy=maxjmini,kμxe1xi,μxe2xi,μxe3xi,μxe4xi

For​ y=112

μy=μ112=maxjmini,kμxe1x1,μxe2x1,μxe3x1,μxe4x1

=maxjmini,k0,332 Λ 0 Λ 0,332 Λ 0 = maxj0.  Hence μ112=0

For y=113,5

μy=μ113,5=maxjmini,kμxe1x1,μxe2x2,μxe3x1,μxe4x1μxe1x1,μxe2x1,μxe3x2,μxe4x1μxe1x1,μxe2x1,μxe3x1,μxe4x2μxe1x2,μxe2x1,μxe3x1,μxe4x1

=maxjmini,k0,332 Λ 0,666 Λ 0,332 Λ 0 0,332 Λ 0 Λ 1 Λ 00,332 Λ 0 Λ 0,332 Λ 0,3320,666 Λ 0 Λ 0,332 Λ 0  =maxj0000. Hence μ113,5=0

For y=114,5

μy=μ114,5=maxjmini,kμxe1x1,μxe2x3,μxe3x1,μxe4x1μxe1x1,μxe2x1,μxe3x3,μxe4x1μxe1x1,μxe2x1,μxe3x1,μxe4x3μxe1x3,μxe2x1,μxe3x1,μxe4x1

=maxjmini,k0,332 Λ 1 Λ 0,332 Λ 0 0,332 Λ 0 Λ 0,332  Λ 00,332 Λ 0 Λ 0,332 Λ 11  Λ 0 Λ 0,332 Λ 0 =maxj0000. From whereμ114,5=0

For y=115

μy=μ115=maxjmini,kμxe1x1,μxe2x1,μxe3x2,μxe4x2μxe1x1,μxe2x2,μxe3x1,μxe4x2μxe1x1,μxe2x2,μxe3x2,μxe4x1μxe1x2,μxe2x1,μxe3x2,μxe4x1μxe1x2,μxe2x2,μxe3x1,μxe4x1

28/09,5/30,5/32 /e1

28/e2                                                          29,5/e2                                                                     30,5/e2                                                                32/e2

28/e3   29,5/e3   30,5/e3   32/e3           28/e3           29,5/e3       30,5/e3     32/e3              28/e3      29,5/e3   30,5/e3   32/e3                                28/e3      29,5/e3     30,5/e3     32/e3

Pour e4

29,5    32      29,5    32     29,5         32    29,5     32    29,5   32     29,5        32    29,5      32   29,5    32    29,5   32   29,5      32  29,5     32    29,5      32   29,5     32      29,5     32      29,5       32   29,5     32

For e 1

For e 2

For e 3

28 30.5 28 30.5 28 30.5 28 30.5 28 30.5 28 30.5 28 30.5 28 30.5 28 30.5 28 30.5 28 30.5 28 30.5 28 30.5 28 30.5 28 30.5 28 30.5

Fig.3. Characteristic tree of E(x)for e j

Starting from xej=28, each path found gives us a value of ywhich may or may not resemble each other with different degrees of belonging.

For example, the first path gives us; y= j=14xej=xe1+xe2+xe3+xe4=28+28+28+28=112

After several paths per iteration, we have a total of 64 paths; we can conclude with this path: For y=128

μy=μ128=maxjmini,kμxe1x4,μxe2x4,μxe3x4,μxe4x4

=maxjmini,k0 Λ 0,332 Λ 0,332 Λ 0,666.

= maxj0

Henceμ128=0

The summons is valid;

y= j=14xe4=xe1+xe2+xe3+xe4.

.=1120;113,50;114,50;1150,332;;1160;116,50,332;1170,332;1180,332;119,50,332;1200,666;1211;1220,332;1240,666;126,50,332;1280

Of all our different fuzzy sums, the one with the most precise degree of membership is y= j=14xe4=1211because the degree of membership is 1 for the sum of 121. The mathematical expectation is given by the expression;

Exej=1 4 j=14xe4

= 280;28,3750;28,6250;28,750,332290,332;29,1250,332;29,250,332;29,50,332;29,8750,332;300,666;30,251;30,50,332;310,666;31,6250,332;320

From this we can form a fuzzy triangular number of fuzzy mathematical expectation, thus we have;

Exej=28,625;30,25;32

7. Concept of fuzzy variance

Fuzzy variance is a dispersion parameter of a fuzzy distribution (a distribution made up of fuzzy numbers) of a random variable.

Noted  Vxej . It is expressed by the expression:

.Vxej = γo=1nj=1ixej-xej2

With xejis the fuzzy mean

Starting from the illustration, our fuzzy mathematical expectation has a mode of 30.25Kv because the value of the mean whose membership function is 1. We will calculate the fuzzy variance for each xej.

| xej-xej x | xe1-xej | xe2-xej | xe3-xej | xe4-xej |
| --- | --- | --- | --- | --- |
| 28kV | -2,25 Kv/0.332 | -2,25 Kv/0 | -2,25 Kv/0.332 | -2,25 Kv/0 |
| 29.5kV | -0,75Kv/0.666 | -0,75Kv/0.666 | -0,75Kv/1 | -0,75Kv/0.332 |
| 30.5kV | 0,25Kv/1 | 0,25Kv Kv/1 | 0,25Kv Kv/0.332 | 0,25Kv Kv/1 |
| 32kV | 1,75Kv/0 | 1,75Kv/0.332 | 1,75Kv/0.332 | 1,75Kv/0.666 |

Table 6. Different voltage xej-xejof

Starting from the expression

Vxej =1nj=1ixej-xej2

Gold yj,i=xej-xej

Thus, the expression for variance can be written;

Vxej =1nj=1iyj,i2

We have the distribution

| yj,i2 x | xe1-xej2 | xe2-xej2 | xe3-xej2 | xe4-xej2 |
| --- | --- | --- | --- | --- |
| 28kV | 5,0625 Kv/0.332 | 5,0625 Kv/0 | 5,0625 Kv/0.332 | 5,0625 Kv/0 |
| 29.5kV | 0.5625 kV / 0.666 | 0.5625 kV / 0.666 | 0.5625 Kv /1 | 0.5625 kV / 0.332 |
| 30.5kV | 0.0625 kV /1 | 0.0625 kV /1 | 0.0625 kV / 0.332 | 0.0625 kV /1 |
| 32kV | 3.0625 Kv /0 | 3.0625 kV / 0.332 | 3.0625 kV / 0.332 | 3.0625 kV / 0.666 |

Table 7. Different voltages ofxe1-xej2

Our distribution is a fuzzy number distribution, which always requires that we be able to associate each value with its degree of membership. Referring to the membership function, and using the part tree to calculate the fuzzy mean, we will now use the tree to calculate the fuzzy variance. Thus, here are the different sums of the expression j=14xej-xe2 we will find using the membership function. This will allow us to determine the fuzzy variance.

Thus, the result of the expression yis;

y(ej)= j=14xej-x(ei)2.

= 0,250,332;0,751;1,250,666;1,750,666;2 ,250,332;3,250,332;3,75/0,666;4,250,666;4,75/0,666;5,250,332;5,750,332;6,250,332;6,750,332;7,250,332;8,250,332;8,750,332;9,250,332;9,750,332;10,250,332;10,750,332;11,250,332;11,750,332;12,250;13,250,332;13,750,332;14,250,332;15,250;15,750;16,250;18,250;20,250,332

But the variance is ;Vxej =1 4 j=14xej-x(ei)2

= 0,06250,332;0,18751;0,31250,666;0,43750,666;0,56250,332;0,81250,332;0,9375/0,666;1,06250,666;1,1875/0,666;1,31250,332;1,43750,332;1,56250,332;1,68750,332;1,81250,332;2,06250,332;2,18750,332;2,31250,332;2,43750,332;2,56250,332;2,68750,332;2,81250,332;2,93750,332;3,06250;3,31250,332;3,43750,332;3,56250,332;3,81250;3,93750;4,06250;4,56250;5,06250,332

Of all our different fuzzy variances, the one with a membership degree of 1 is of interest to us , Vxej =0,18751 . We are dealing with a unimodal fuzzy variance, which explains why Vxej it is a triangular number.

8. Fuzzy autocovariance and autocorrelation function

Fuzzy autocovariance function

Autocovariance is a function that studies the relationship between the process xtand the process xt-hshifted by hstep ( ) xt-h). Hence, the autocovariance function is represented by the expression :

γh= γh=covxt,xt-h

autocovariance function is said to be fuzzy if it studies the relationship that exists between the process xejand the process xej-h. Hence, the processes are forms of fuzzy numbers.

Exej.xej-i=γi

Which gives:

Exejxej-1= 1nj=1nxej-xejxej-1-xej

Let's ask  z=j=1nxej-xejxej-1-xej

From our illustration, we can calculate the autocovariance of our fuzzy distribution using Zadet 's extension method for multiplication and addition.

|  | xej | xej-xej |  | xej-1 | xej-1-xej | z |
| --- | --- | --- | --- | --- | --- | --- |
| xe1 | 28kv | -2,25Kv |  |  |  |  |
|  | 29.5kv | -0,75Kv |  |  |  |  |
|  | 30.5kv | 0,25Kv |  |  |  |  |
|  | 32kv | 1,75Kv |  |  |  |  |
| xe2 | 28kv | -2,25Kv | xe1 | 28kv | -2,25Kv | 5.0625kV |
|  | 29.5kv | -0,75Kv |  | 29.5kv | -0,75Kv | 0.5625kV |
|  | 30.5kv | 0,25Kv |  | 30.5kv | 0,25Kv | 0.0625kV |
|  | 32kv | 1,75Kv |  | 32kv | 1,75Kv | 3.0625kV |
| xe3 | 28kv | -2,25Kv | xe2 | 28kv | -2,25Kv | 5.0625kV |
|  | 29.5kv | -0,75Kv |  | 29.5kv | -0,75Kv | 0.5625kV |
|  | 30.5kv | 0,25Kv |  | 30.5kv | 0,25Kv | 0.0625kV |
|  | 32kv | 1,75Kv |  | 32kv | 1,75Kv | 3.0625kV |
| xe4 | 28kv | -2,25Kv | xe3 | 28kv | -2,25Kv | 5.0625kV |
|  | 29.5kv | -0,75Kv |  | 29.5kv | -0,75Kv | 0.5625kV |
|  | 30.5kv | 0,25Kv |  | 30.5kv | 0,25Kv | 0.0625kV |
|  | 32kv | 1,75Kv |  | 32kv | 1,75Kv | 3.0625kV |

Table 8. First-order autocovariance

We will determine the degree of membership for each value found relative to the fuzzy number xej withj=1,…,4

covxej,xej-1=Exej,xej-1= 1nj=1nxej-xejxej-1-xej.

AndExej,xej-1=γ1

Starting from the empirical expression of the autocovariance function with respect to the result, we have;

covxej,xej-1=1nj=1nxej-xejxej-1-xej.

= 14.15,1875;0,13,6875;0,11,1875;0,10,6875;0,10,1875;0,9,1875;0,8,6875;0,8,1875;0,6,6875;0,332,6,1875;0,5,6875;0,5,1875;0,4,1875;0,332,3,6875;0,3323,1875;0,332,1,6875;0,332,1,1875;0,332,0,6875;0,332

Hence covxej,xej-1=3,7968;0,3,4218;0,2,7968;0,2,6718;0,2,5468;0,2,2968;0,2,1718;0,2,0468;0,1,6718;0,332,1,5468;0,1,4218;0,1,2968;0,1,0468;0,332,0,9218;0,3320,7968;0,332,0,4218;0,332,0,2968;0,332,0,1718;0,332

When we consider our results, there is no value with a membership degree of 1. The highest degree is 0.332. Therefore, we will take the partial autocovariances with a membership degree of 0.332 to calculate the different autocorrelation functions.

γ1=(1,67;0,332,1,04;0,332,0,09;0,332,0,79;0,332,0,42;0,332,029;0,332,0,17;0,332)

When we take into account our results, there is no value with a membership degree of 1. The highest degree is 0.332. Hence, we will take the partial autocovariance with a membership degree of 0.332 to calculate the different autocorrelation functions.

8.2. Fuzzy autocorrelation function

The autocorrelation function is a function that studies the relationship between the process xtand the process xtshifted by a hstep size ( ) xt-h). Hence, the autocorrelation function is represented by the expression :

.φx(ej)=γx(ej)γo

Gold γo=0,18751

- φx(e1)=γx(e1)γo= (1,6718/0,332)(0,1875/1)= 8,9163

Searching for the membership function,

μφ(1)=maxj=1minj0,332Λ1=maxj=1(0,332)=0,332

- φx(ej=2)=γx(e2)γo=( 1,0468/0,332)(0,18751 )= 5,5829

Searching for the membership function,

μφ(2)=maxj=2minj0,332 Λ 1=maxj=2(0,332)=0,332

- φx(ej=3)=γx(e3)γo= (0,9218/0,332)(0,1875/1)=4,9162

Searching for the membership function ,

μφ(3)=maxj=3minj0,332Λ1=maxj=3(0,332)=0,332

- φx(4)=γx(e4)γo=(0,7968/0,332)(0,1875/1)=4,2496

Searching for the membership function,

μφ(4)=maxj=4minj0,332Λ1=maxj=4(0,332)=0,332

- φx(ej=5)=γx(e5)γo=. (0,4218/0,332)(0,3125/1)= 2,2496

Searching for the membership function,

μφ(21)=maxj=21minj0,332Λ1=maxj=21(0,332)=0,332

- φx(ej= 6)=γx(e6)γo=( 0,2968/0,332)(0,31251 )= 1,5829

Searching for the membership function,

μφ(22)=maxj=22minj0,332 Λ 1=maxj=22(0,332)=0,332

- φx(ej=  7)=γx(e7)γo= (0,1718/0,332)(0,3125/1)=0, 9163

Searching for the membership function,

μφ(23)=maxj=23minj0,332Λ1=maxj=23(0,332)=0,332

We observed that all the different autocorrelation functions have a membership degree greater than 0.332. Our model is made up of partial autocorrelation functions because there is no total autocorrelation function, i.e., no autocorrelation function whose membership degree is 1.

# 9. Fuzzy parameter estimation of the fuzzy autoregressive model

Starting from the fuzzy autocorrelation function, we can determine the fuzzy parameter of the fuzzy autoregressive part. The expression that allows us to calculate it is as follows, while respecting the membership function of each autocorrelation function, which will allow us to determine the membership function of each parameter found for each model and jexplains the different partial model that we will estimate. Now, we have a fuzzy model with a single fuzzy variance that has a membership degree of 1. This explains the number of lags in the process, which forms the partial model;

φx(ej)=ϕ1φx(ej-1)⟺ϕ1=φx(ej)φx(ej-1)

We will determine the value of φx(e0)by the following expression for all the different coefficients of the model.

φx(ej=0)=γx(e0)γo= 1  avec μ(γo)x=1

For j=1: φx(ej)=ϕ1φx(ej-1)⟺ϕ11=φxe1φxe0=8,91630,33211

Henceϕ11=8,91630,332

For j=2: φx(ej)=ϕ1φx(ej-1)⟺ϕ12=φxe2φxe1=5,58290,3328,91630,332

Henceϕ12=0,62610,332

For j=3: φx(ej)=ϕ1φx(ej-1)⟺ϕ13=φxe3φxe2=4,91620,3325,58290,332

Henceϕ13=0,88060,332

For j=4: φx(ej)=ϕ1φx(ej-1)⟺ϕ14=φxe4φxe3=4,24960,3324,91620,332

Henceϕ14=0,86440,332

For j=5: φx(ej)=ϕ1φx(ej-1)⟺ϕ15=φxe5φxe4=2,24960,3324,24960,332

Henceϕ15=0,52940,332

For j=6: φx(ej)=ϕ1φx(ej-1)⟺ϕ16=φxe6φxe5=1,58290,3322,24960,332

Henceϕ16=0,70360,332

For j=7: φx(ej)=ϕ1φx(ej-1)⟺ϕ17=φxe7φxe6=0,91630,3321,58290,332

Henceϕ17=0,57890,332

autocovariance functions of the fuzzy autoregressive process of order

1. The expression of the model;

xet-ϕμejxet-1=wt

Which gives;                        xet=wt+ϕμejxet-1

xet=wt+ϕμejxet-1⟺xet=8,9163μ0,332xet-1+wtxet=0,6261μ0,332xet-1+wtxet=0,8806μ0,332xet-1+wtxet=0,8644μ0,332xet-1+wtxet=0,5294μ0,332xet-1+wtxet=0,7036μ0,332xet-1+wtxet=0,5789μ0,332xet-1+wt

10. Conclusion

Based on our study, we estimated the fuzzy stochastic parameters and the coefficient of the fuzzy model. Among the estimated fuzzy stochastic parameters, the expected value and variance are triangular fuzzy subsets of mode 1, while the fuzzy autocovariance and fuzzy autocorrelation are triangular fuzzy subsets of mode 0.332. This explains the partiality of our fuzzy autoregressive process model.

We concluded that the fuzzy autocovariance function , fuzzy autocorrelation function, and fuzzy variance are essential stochastic parameters for estimating the coefficients of a fuzzy model of the autoregressive process, which even explains the stationarity of a stochastic process. In the fuzzy environment, we distinguish two types of fuzzy autoregressive process models; this distinction takes into account the degree to which fuzzy stochastic parameters are included. Thus, we have:

- The partial model of the autoregressive process and

- The complete model of the autoregressive process

In the classical model, parameter estimation resulted in a single model with the estimated parameter, whereas in a fuzzy model, estimation leads to several models of the same order, with the parameter and coefficient of the model being fuzzy numbers and their degree of membership. The fuzzy model allows us to estimate any practical reality adapted to our realities.

References

- Basak , P., Linero , A., Sinha, D., & Lipsitz , S. (2020). Semiparametric Analysis of Clustered Survival Data Using Bayesian Additive Regression Trees .

- Chipman , H., George , E., & McCulloch , R. (2010). BART: Bayesian Additive Regression Trees.

- Hyndman , R.J., & Athanasopoulos , G. (2021). Forecasting : Principles and Practice (3rd ed .).

- Velenik , Y. (2018). Probability and Statistics. University of Geneva.

- Chopin, N. (2012). Introduction to stochastic processes.

- Ross, TJ (2010). Fuzzy Logic with Engineering Applications (read notes extracts ).

- Rubenthaler , S. (2012). Stochastic processes and modeling.

- Pagani , C., Sadaoui, A., & Chabot, S. (2016). Introduction to fuzzy logic.

- Zhou, L. (2024). Rules and Algorithms for Objective Construction of Fuzzy Sets.

- Koklu , A., Guven , Y., & Kumbasar , T. (2024). Efficient Learning of Fuzzy Logic Systems for Large- Scale Data Using Deep Learning.

- Gerdes , W., & Acar , E. (2024). Integrating Fuzzy Logic into Deep Symbolic Regression .

- Locatelli, M., et al. (2024). Reinforcement Learning and Fuzzy Logic Modelling for Optimal Dynamic Treatment .

- Li, Y., et al. (2024). Evaluating Fuzzy Reasoning of Generalized Quantifiers in Large Language Models .

- Chen, G.H. (2024). An Introduction to Deep Survival Analysis Models for Predicting Time-to-Event Outcomes .

- Su , M., & Wang , R. (2025). Moment- Assisted Subsampling Method for Cox Proportional Hazards Model with Large- Scale Data.

- Si, W., et al. (2025). Training-Set Conditionally Valid Prediction Sets with Right- Censored Data.

- Campanella, G., et al. (2025). A Flexible Deep Learning Framework for Survival Analysis with Medical Data.

- Zu Westram , MM, et al. (2025). Solar Flare Forecasting Utilizing Deep Survival Analysis .

- Abraich , A. (2025). TV- SurvCaus : Dynamic Representation Balancing for Causal Survival Analysis .

- Ouifak , H., et al. (2024). Insights into the Potential of Fuzzy Systems for Medical AI Applications.

- Suzuki, A. (2024). Fuzzy Logic Systems for Healthcare Applications.

- Saatchi, R. (2024). Fuzzy Logic Concepts, Developments and Implementation .

- Anuar , NH, Razak , TR, & Kamis, NH (2024). Advancing Fuzzy Logic : A Hierarchical Fuzzy System Approach .

- Huang , L., Xing , Y., Lin , Q., Ruan , S., & Feng , M. (2024). EsurvFusion : An Evidential Multimodal Survival Fusion Model Based on Gaussian Random Fuzzy Numbers .

- Aziz, NR, & Hussein, MMF (2025). Fuzzification Techniques for Survival Time Estimation Using Weibull Distribution.

- Baral , S., et al. (2025). Hybrid Deep Learning and Fuzzy Inference System for Pneumonia Severity Classification.

- Małolepsza , O., Mikołajewski , D., & Prokopowicz , P. (2025). Adaptation of Fuzzy Systems Based on Ordered Fuzzy Numbers : A Review of Applications and Development Prospects.

- DiMari , F., et al. (2025). Relative Survival Cure Models Based on Generalized Additive Models and Neural Networks.

- Konstantinov , AV, Efremenko , VA, & Utkin , LV (2025). Survival Analysis as Imprecise Classification with Trainable Kernels .

- Liu , K., Ma , J., & Lai , E. MK. (2025). Dynamic Fuzzy Rule and Attribute Management Framework for High- Dimensional Data.

- Zemlītis , M., et al. (2025). Proceedings of the International Symposium on Fuzzy Sets and Applications.

- Singh, R., et al. (2025). Fuzzy Sets and Fuzzy Logic : Concepts, Trends and Applications.