Original Research Article

WATER QUALITY ASSESSMENT OF BEGUM TALAB LAKE, VIJAYAPURA DISTRICT FOR DRINKING PURPOSE USING WQI (WATER QUALITY INDEX)

ABSTRACT

The present study assessed the drinking water quality of Begum Talab Lake, located in Vijayapura district, Karnataka, using the Water Quality Index (WQI) approach. Composite surface water samples were collected monthly from July 2024 to June 2025 and analyzed for major physico-chemical parameters following APHA (1998) methods. The average pH was 7.77, remaining within the BIS desirable limit (6.5–8.5). Dissolved oxygen (DO) ranged from 4.4 to 7.6 mg/l, with an average of 5.45 mg/l. Biological oxygen demand (BOD) varied between 1.7 and 5.36 mg/l (mean 3.79 mg/l), while chemical oxygen demand (COD) ranged from 11 to 24 mg/l (mean 19.12 mg/l). Alkalinity (96 mg/l) and hardness (105.17 mg/l) indicated moderately hard water. Total dissolved solids (TDS) and total suspended solids (TSS) averaged 228 mg/l and 39 mg/l, respectively, both within permissible limits. Nitrate concentration remained low, with an average of 0.14 mg/l. The computed overall WQI was 97.59, classifying the lake water as “Excellent” according to CCME (2005). The findings indicate that Begum Talab Lake water is suitable for drinking purposes and requires minimal treatment, though regular monitoring is recommended for sustainable management.

INTRODUCTION

Freshwater is required for sustenance of life on Earth. Freshwater can be found in surface water bodies such as lakes, rivers, ponds and canals. Lake is a large depression in Earth, filled with slowly moving or standing water (Krishne Gowda, 2007). Lakes cover only a very small portion of earth’s surface. Freshwater lakes are the wealth of a nation, being a source of water for both irrigation and drinking. The surface water is an integral part of natural environment and its quality is very sensitive and critical issue in many countries. Also, with an increased understanding of the importance of drinking water quality to public health and raw water quality to aquatic life, there is a great need to assess surface water quality (Ouyang 2003).

The term “water quality” includes the water column and the physical channel required to sustain aquatic life. The goal of the federal Clean Water Act, “To protect and maintain the chemical, physical and biological integrity of the nation's waters,” establishes the importance of assessing both water quality and the habitat required for maintaining other aquatic organisms. Water, the precious gift of nature to human being, is going to be polluted day-by-day with increasing urbanization (Bharati et al., 2016).

The use of water quality indices (WQI) is a simple practice that overcomes many of the previous mentioned problems and allows the public and decision makers to receive water quality information (AOAC) (1995). The major tool of pollution profile studies is the water quality index (WQI). Horton proposed the first formal WQI in the literature in 1965 (Ott, 1978). Water Quality Index (WQI) is regarded as one of the most effective way to communicate water quality (Pradhan, et al., 2001). In a number of nationwide studies, water quality of different natural resources was assessed on the basis of calculated water quality indices (Rajmohan, 2003).

The quality of water in lakes, reservoirs, rivers are determined by physico-chemical and biological parameters. Water quality index provides overall water quality at a certain location and time based on multiple water quality parameters. Water quality monitoring is one of the key tools, to identify and keep a check on the pollution status and ensure about the efficiency of management plans (Singh and Jayakumar 2016). Hence monitoring of these aquatic resources is crucial for sustainable management. Water Quality Indices (WQI) are broadly classified into two types, physico chemical indices and biological indices. Physico chemical indices are determined from the values of various chemical and physical parameters in water samples, while biological indices are derived from biological data. In this study, the WQI of the area has been calculated using hydrochemical parameters. The objective of the present investigation was to provide information on the physio-chemical parameters of Begum Talab Lake and its suitability for human consumption based on computed water quality index (WQI).

MATERIALS AND METHODS

Study area

Begum Talab is a village located in the Vijayapuraaa, (16° 794631 N latitude and 75° 707293 E longitude) of Karnataka state, India (Fig. 1). Built in 1651 by Mohammad Ali Shah and named after Adil Shah’s queen Jahan Begum, this reservoir was supervised by commander Afzal Khan. It played a vital role in supplying water to Vijayapuraaa's population. Surrounded by lush greenery and lively activities along its perimeter, the site is both scenic and culturally vibrant. Along with other Bijapur landmarks, it is on UNESCO’s tentative list under Monuments and Forts of the Deccan Sultanate.

Figure 1: Map of study area (Begum Talab Lake)

Composite surface water samples were collected monthly from Begum Talab Lake (Fig. 1) between July 2024 and June 2025.  Physico chemical parameters, including pH, DO, BOD, COD, alkalinity, hardness, TDS, TSS, and nitrate, were analysed. pH was measured on site, while biochemical oxygen demand was determined using the Winkler method in separate 300 mL bottles. Samples were transported to the laboratory in dark ice boxes to prevent exposure to sunlight. Analyses of physico chemical variables followed the Standard Methods (APHA, 1998). The Water Quality Index (WQI) was calculated using the drinking water quality standards recommended by the World Health Organization.

Calculation of WQI

WQI=ÓQnwÓwn

Where:

- Qn = Water quality rating

Qn=100×Vn-ViSn-Vi

- Vn = Actual value of the parameter in the water sample

- Vi = Ideal value ((0 for all parameters except pH and DO which is 7.0 and 14.6 mg/l,       respectively)

- Sn = Standard value

- wn = Unit weight of the parameter

wn=KSn

- K = Constant, calculated as

K=11S1+1S2+…1Sn

Classification of WQI according to CCME (2005).

RESULTS AND DISCUSSION

Physico-chemical analysis of water quality

Several studies have assessed lake water quality using physico-chemical parameters (Giriyappanavar et al., 2013).

pH

The overall quality of water is largely defined by its chemical characteristics. pH, for instance, is a key indicator of water suitability for various uses. In Begum Talab Lake, the average pH was recorded at 7.82, remaining alkaline throughout the study period, with a maximum value of 8.4 observed in September. These values fall within the desirable limits prescribed by BIS. Comparable findings were reported by Kodom et al. (2018) in a tropical lake in Ghana and by Singh et al. (2020) in Fateh Sagar Lake.

Dissolved oxygen (DO)

Dissolved oxygen (DO) in water bodies is shaped by factors such as temperature, photosynthesis, wind, respiration, and pollution. Its levels vary daily, seasonally, and with temperature changes (Wavde and Arjun, 2010). Dissolved oxygen (DO) levels ranged between 4.4 and 7.6 mg/l, peaking in February (summer) and reaching a minimum in June. The present findings align with those of Kodom et al. (2018) on Volta Lake, Ghana, but contrast with Kangabam et al. (2017) on Loktak Lake, Manipur.

Biological Oxygen Demand (BOD) and Chemical Oxygen Demand (COD)

The BOD test assesses the presence of organic matter in water by measuring the oxygen required by aquatic microbes to decompose biodegradable material (Sharma et al., 2013). Biological Oxygen Demand (BOD), which reflects the extent of organic matter pollution (Kumar and Bahadur, 2009), varied from 1.69 mg/l in May to 5.05 mg/l in December. Chemical oxygen demand (COD) measures the total amount of organic matter in water, both biodegradable and non-biodegradable (Khan et al., 2014). Chemical Oxygen Demand (COD) values ranged between 11.36 and 24.12 mg/l, with the lowest value recorded in November and the highest in June. Since COD provides faster results than BOD, it is generally preferred (Sharma et al., 2013).

Total alkalinity

Total alkalinity reflects the water’s buffering capacity, helping to stabilize pH (Lodh et al., 2014). It measures the ability to neutralize acids and is generally not considered a pollutant (Sharma and Kumar, 2017). The average alkalinity of Begum Talab Lake ranged between 70.0 and 120 mg/l, consistent with observations from Almatti Reservoir, Karnataka (Hulyal and Kaliwal, 2011). The high alkalinity observed in summer resulted from increased decomposition, which released CO₂ and added carbonate and bicarbonate ions, thereby raising alkalinity levels (Verma et al., 2012).

Water hardness

Water hardness primarily results from calcium and magnesium cations. Water hardness varied from 75 to 125 mg/l, with an average of 115 mg/l, classifying the lake water as moderately hard according to Sawyer (1960). Usman et al. (2018) observed higher water hardness in Mumbai’s urban lakes during summer and lower levels in winter, while Manzoor et al. (2017) and Tyor & Chawla (2012) reported much lower hardness values in Sukhna Lake.

Total dissolved solids (TDS) and total suspended solids (TSS)

According to WHO (2011) guidelines, drinking water is considered palatable when total dissolved solids (TDS) are below 600 mg/l, while concentrations above 1000 mg/l are deemed unappealing. Total dissolved solids (TDS) and total suspended solids (TSS) were recorded between 20–320 mg/l and 4–64 mg/l, respectively. For drinking water, BIS 10500 (2012) prescribes a desirable limit of 500 mg/l for TDS, extendable up to 2000 mg/l in the absence of alternative sources (Dubey et al., 2014). Thus, the observed TDS and TSS values fall within acceptable limits.

Nitrate

Inorganic nitrogen is the most common form in natural waters and serves as a key nutrient driving the growth of hydrophytes and aquatic algae (Lodh et al., 2014). Nitrate concentrations ranged from 0.01 to 0.6 mg/l, with an average of 0.25 mg/l. Similar observations were noted by Verma et al. (2012). During the monsoon, nitrogen-rich floodwaters from the catchment introduced contaminants such as sewage, elevating nitrate levels consistent with the present study. In contrast, Bhat and Pandit (2014) reported lower nitrate concentrations in Wular Lake, Kashmir during summer. Decomposing plant matter and agricultural runoff elevate nitrogen levels in surface waters. Nitrate, a key indicator of organic pollution, originates from fertilizers, rainfall, crop residues, decaying organic waste, and septic systems (Khan et al., 2014).

Water quality assessment of Begum Talab lake

Table 1: WQI classification by CCME (2005).

| WQI | Quality | Description |
| --- | --- | --- |
| 95–100 | Excellent | Water quality is protected with virtual absence of threat or impairment; conditions very close to natural or desirable levels. |
| 89–94 | Very good | Water quality is protected with a slight presence of threat or impairment; conditions close to natural or pristine levels. |
| 80–88 | Good | Water quality is protected with only minor degree of threat or impairment; conditions depart from natural or desirable levels. |
| 60–79 | Fair | Water quality is usually protected but occasionally threatened or impaired; conditions sometimes depart from natural or desirable levels. |
| 45–59 | Poor (Marginal) | Water quality is frequently threatened or impaired; conditions sometimes often depart from natural or desirable levels. |
| 0–44 | Very poor | Water quality is almost always threatened or impaired; conditions sometimes usually depart from natural or desirable levels (Unsuitable for drinking). |

Table 2: WQI of Begum Talab lake water (all values except pH are in mg/l).

| Parameter | Actual Average Value | Standard Value | Recommended Agency | 1/Sn | Wn | Qn | QnWn |
| --- | --- | --- | --- | --- | --- | --- | --- |
| pH | 7.82 | 6.5–8.5 | ICMR / BIS | 0.117 | 54.71 | 0.1805 | 9.87 |
| DO | 5.40 | 5 | ICMR / BIS | 0.200 | 104.50 | 0.3068 | 32.06 |
| BOD | 3.95 | 5 | WHO / ICMR | 0.200 | 79.00 | 0.3068 | 24.24 |
| COD | 18.30 | 10 | WHO | 0.100 | 183.00 | 0.1534 | 28.09 |
| Alkalinity | 110.00 | 200 | BIS | 0.005 | 55.00 | 0.00767 | 0.42 |
| Hardness | 115.00 | 200 | WHO | 0.005 | 57.50 | 0.00767 | 0.44 |
| TDS | 240.00 | 500 | WHO / BIS | 0.002 | 48.00 | 0.00307 | 0.15 |
| TSS | 40.00 | 500 | WHO | 0.002 | 8.00 | 0.00307 | 0.02 |
| Nitrate | 0.25 | 50 | WHO | 0.020 | 0.50 | 0.03068 | 0.02 |

Overall WQI = 92.16

The Water Quality Index (WQI) condenses complex analytical data into a single value that subjectively represents overall water quality. This rating system enhances simplicity and public understanding. Various WQI models have been proposed in the literature, with comparative evaluations reported (Cude, 2001). Depending on its intended application, a WQI may be tailored for specific water bodies or designed as a general index applicable to all waters intended for human consumption.

Water Quality Index (WQI)

Inland ecosystems play a vital role in supplying drinking water, recharging groundwater, maintaining water quality, recycling nutrients, and removing toxins. In the present study, the calculated WQI of Begum Talab Lake was 92.16 (Table 2), indicating very good water quality. This score reflects conditions that are virtually free from threats or impairments, closely resembling natural or desirable levels (CCME, 2005). In comparison, Thakor et al. (2011) reported a WQI below 75 for Pariyej Lake, Gujarat; Bharamal and Korgaonkar (2014) recorded 75.82 for Palnekond Reservoir in Sawantwadi; and Alaka (2014) observed 76.27 in Bargaon Reservoir, Maharashtra.

Conclusion

The present study assessed the drinking water quality of Begum Talab Lake using physico chemical parameters and the Water Quality Index method. All the analysed parameters such as pH, dissolved oxygen, biological oxygen demand, chemical oxygen demand, alkalinity, hardness, total dissolved solids, total suspended solids, and nitrate were found within permissible limits of drinking water standards. The calculated Water Quality Index value of 92.16 indicates that the lake water falls under the “Very Good” category. This shows that the water quality is largely protected and close to desirable conditions, with only minor variations. Therefore, Begum Talab Lake water is suitable for drinking purposes with minimal treatment. However, regular monitoring and proper management are recommended to maintain its quality in the future.

REFERENCE

- Alaka P A (2014) An assessment of water quality of Borgaon reservoir in Sangli district of Maharashtra, India. Int. Res. J. Environ. Sci. 3(5), 48-53.

- APHA (1998) Standard methods for the examination of water and waste water. (20 ed.). American Public Health Association, Washington D.C.

- Bharamal D L and Korgaonkar D S (2014) Determination of water quality index of Palnekond reservoir in Sawantwadi Taluka, Sindhudurg. Int. J. Curr. Microbiol. App. Sci. 3(6), 819-830.

- Bharathi, H. R., Manjappa, S., Suresh, T., & Suresh, B. (2016). Evaluation of water quality index of water bodies Channarayapatna taluk, Karnataka region, India. International Journal of Applied Sciences and Biotechnology, 4(4), 475-482.

- Bhat SA, Pandit AK (2014). Surface water quality assessment of Wular Lake, a ramsar site in Kashmir Himalaya, using discri minant analysis and WQI. J. Ecosyst. 2014

- CCME (Canadian Council of Ministers of the Environment) (2005) Standard Methods for the examinations of water and wastewater, American Public Health Association, Washington DC.

- Cude C G (2001) Oregon water quality index: A tool for evaluating water quality management effectiveness. J. American Water Research Assoc. 37, 125-137.

- Dubey R K, Hussain J, Malhotra N and Mehta A (2014) Ground water quality and water quality index of Dwarka district of National capital of India. I J R E T 3(4), 85-93.

- Garg R K, Rao R J and Saksena D N (2009) Water quality and conservation management of Ramsagar reservoir, Datia, Madhya Pradesh. J. Exp. Biol. 30(5), 909-916.

- Giriyappanavar B S and Patil R R (2013) Water Quality Assessment of Gullava Reservoir of Belgaum District Using NSF-WQI. I J S R. 2(4), 71-72.

- Gowda, K., & Sridhara, M. V. (2007). Conservation of tanks/lakes in the Bangalore metropolitan area. Management of Environmental Quality: An International Journal, 18(2), 137-151.

- Hulyal S B and Kaliwal B B (2011) Seasonal Variations in Physico Chemical Characteristics of Almatti Reservoir of Bijapur district, Karnataka State. I J E P 1(1), 58-67.

- Kangabam RD, Bhoominathan SD, Kanagaraj S, Govindaraju M (2017) Development of a water quality index (WQI) for the Loktak Lake in India. Appl Water Sci 7(6):2907–2918

- Khan, A.A., Gaur, R.Z., Mehrotra, I., Diamantis, V., Lew, B. & Kazmi, A.A. (2014). Performance assessment of different STPs based on UASB followed by aerobic post treatment systems. J Environ Health Sci Eng., https :// doi.org/10.1186/2052-336x-12-43

- Kodom K, Onoyinka AA. Mkude IT, Otu SA, Yeboah JO (2018). Physicochemical parameters assessment and seasonal variations of surface water quality of the volta lake in Ghana

- Kumar A Bahadur (2009) Physico-chemical Studies on the Pollution Potential of River Kosi at Rampur (India). World J. Agricult. Sci. 5(1), 1-4.

- Lodh R, Paul R, Karmakar B, Das MK (2014) Physicochemical stud ies of water quality with special reference to ancient lakes of Udaipur City, Tripura, India. Int J Sci Res Publ 4(6):1–9

- Ott WR (1978) Environmental Indices: Theory and Practice, Ann Arbor, MI (USA): Science Publishers Inc.

- Ouyang, Y. (2005). Evaluation of river water quality monitoring stations by principal component analysis. Water research, 39(12), 2621-2635.

- Pradhan, S. K., Patnaik, D., & Rout, S. P. (2001). Water Quality Index for the ground water around a phosphatic fertilizer plant. Indian Journal of Environmental Protection, 21(4), 355-358.

- Rajmohan, N (2003) Major correlation in ground water of Kancheepuram region, South India, Indian J. Environ. Hlth. 45(1): 1-5.

- Rajmohan, N (2003) Major correlation in ground water of Kancheepuram region, South India, Indian J. Environ. Hlth. 45(1): 1-5.

- Sawyer C H (1960) Chemistry for sanitary Engineers. McGraw Hill Book Co., New York.

- Sharma RC, Kumar R (2017) Water quality assessment of sacred gla cial Lake Satopanth of Garhwal Himalaya. India Appl. Water Sci. 7(8):4757–4764

- Sharma, P., Khitoliya, R.K. & Kumar, S. (2013). A comparative Sharma, P., Khitoliya, R.K. & Kumar, S. (2013). A comparative study of sewerage treatment plants with different technolo gies in the vicinity of Chandigarh city. J Environ Sci Toxi col Food Technol., 4(5): 113–121.

- Singh RP, Mangottiri V, Pandiyan B (2020) Understanding the vari ability in estimation of water quality of lakes and reservoirs. In IOP Conference Series: Materials Science and Engineering. 955(1):012085

- Singh, A. K., & Jayakumar, S. (2016). Water quality assessment of Kanwar Lake, Begusarai, Bihar, India. Imperial Journal of Interdisciplinary Research, 2(4), 793-803.

- Thakor S J, Bhoi D K, Dabhi H R, Pandya S N and Nikitaraj B C (2011) Water Quality Index (W.Q.I.) of Pariyej Lake Dist. Kheda – Gujarat. Curr. World Environ. 6(2), 225- 231.

- Tyor AK, Chawla D (2012). Survey and study of phytoplankton ecol ogy in Sukhna Lake, Chandigarh, (India)

- Usman A, Dube K, Shukla SP, Salaskar P, Prakash C, Sawant PB, Singh R (2018) Water quality index as a tool for assessment of status of an urban lake of Mumbai. Int J Curr Microbiol App Sci 7(04):520–533

- Verma P, Chandawat D, Gupta U, Solanki H (2012) Water quality analysis of an organically polluted lake by investigating differ ent physical and chemical parameters. Int J Res Chem Environ 2(1):105–111

- Wavde PN, Arjun B (2010) Groundwater quality assessment at Male gaon region of nanded in Maharashtra (India). J Environ Sci Eng 52(1):57–60