Modelling and Analysis of Thin-Layer Drying Kinetics for Freeze-Dried Carrot Slices

ABSTRACT

This study investigated the influence of heating plate temperature on the thin-layer drying of carrot slices and identified the most supported mathematical model for predicting the drying kinetics. Experiments were conducted in a laboratory setup designed to mimic freeze-drying conditions. Carrot slices of thicknesses (2, 4 and 6 mm) were dried at temperatures (40, 50 and 60 °C). The drying data obtained under these conditions were used to analyse moisture removal patterns and to determine the effective moisture diffusivity during the falling-rate period. To characterise the drying process, 5 semi-theoretical and empirical thin-layer models were fitted to the experimental moisture ratio (MR). The suitability and predictive accuracy of each model were evaluated using statistical indicators, including the root mean square error (RMSE), coefficient of determination (R²) and reduced chi-square. All models showed acceptable performance. Among the models assessed, the logarithmic model consistently provided the best fit across the different temperature and thickness combinations. Its higher R² values and lower RMSE and chi-square values indicated a closer agreement between the predicted and observed drying behaviour for thin-layer drying of carrot slices under the tested conditions. These findings can support the optimisation of freeze-drying simulations and contribute to improving the design of drying systems for heat-sensitive foods.

Keywords: Freeze-drying, Drying kinetics, Carrot slices, Moisture diffusivity, Reduced chi-square.

1. INTRODUCTION

Carrot (Daucus carota L.) is one of the most widely consumed vegetables worldwide and ranks as the second most popular vegetable globally. Its roots are highly valued due to their rich composition of bioactive compounds, including β-carotene, vitamin C, carbohydrates, and essential minerals, which contribute to its recognition as one of the healthiest vegetables (Beaudry, C et al. 2003). These nutrients are associated with notable health-promoting effects, particularly due to their antioxidant, anti-inflammatory, and anti-tumour activities (Ahmad, T et al. 2019). Fresh carrot root has 88% water, 10% carbohydrates and 2% protein with vitamins and minerals. Carrots contain very little fat and protein. The carbohydrates consist of starch and sugars such as sucrose and glucose. Pectin is the main form of soluble fibre in carrots. Carrots are a good source of several minerals and vitamins, especially vitamin A, biotin, vitamin K, Potassium and vitamin B6 and folic acid. Carrots contain many plant compounds, but the carotenoids are by far the best known.

In food processing, drying operations have proven effective in preventing the physical, chemical, and biochemical deterioration of raw biological material. The main goal of drying is reducing the water content. In that way, biological material becomes easier to manipulate and less effected to microbial degradation. Removal of moisture from raw material, or wet food product, enables the shelf life’s extension, prolongs the storage time, reduces volume, restrains microbial growth, and inactivates enzymes (Ozkan, I.A et al. 2007, Al-Harahsheh, M et al. 2009, Soysal, Y et al. 2006). In freshly harvested produce such as carrots, the high moisture content accelerates the degradation of phytochemicals due to both enzymatic and non-enzymatic reactions that occur over time. Freeze-drying (lyophilisation) is widely recognised as an effective preservation technique for such products, as it helps retain nutritional quality better than conventional heat-based drying methods. This technique is based on the principle of sublimation, where the water in the product is first frozen and then removed by directly converting ice into vapour without passing through the liquid state. By avoiding liquid water and excessive heat exposure, freeze-drying significantly reduces enzymatic browning and microbial spoilage. Therefore, it is particularly suitable for foods rich in heat-sensitive compounds, especially antioxidants. The freeze-dried material has a slightly altered chemical composition, particularly with vitamins and micro and macro elements. One of the major merits of the freeze-drying system is that the products after trying the freeze-drying method can be kept at normal temperature, and there is no need to keep freeze-drying products in a cool environment.

The drying behaviour of food materials is a highly complex phenomenon involving simultaneous heat transfer and mass movement, primarily through moisture migration. Numerous researchers have attempted to describe and predict this process by developing simplified mathematical models based on heat and mass transfer principles, enabling better understanding and characterisation of drying behaviour for practical and industrial applications. Such simplified mathematical equations are solved to develop predictive models capable of representing real-world drying behaviour. These models serve as valuable tools for decision-makers, enabling the selection of appropriate drying systems, prediction of moisture content and drying time, and optimisation of operating conditions for improved process efficiency. In other words, without mathematical considerations, operators may have to rely on unsystematic and inefficient practices, which may incur extravagant costs, time, and tedious effort (Chukwunonye, C. D et al. 2016, Onwude, D.I et al. 2016).

The dehydration of food materials involves several internal moisture movement mechanisms, including surface diffusion, vapour diffusion and liquid, capillary action within the porous structure. In convective thin-layer drying, where heated air passes over the material, heat and mass transfer occur simultaneously. During this process, moisture migrates from the interior of the solid to its surface, while the resulting vapour is carried away by convective airflow and heat is transmitted primarily through conduction (Akpinar, E et al. 2003). Single layer arrangement of the material is referred as the thin layer, which typically enhances the rate of moisture removal, reduces nutrient loss, and simplifies mathematical modelling. According to (Chakraverty, A et al. 2003), product layers up to 20 cm in thickness can still be classified as thin layers. In such systems, the drying air is generally assumed to maintain uniform temperature and relative humidity throughout the process.

The falling-rate stage of drying, thin-layer drying models assume that the product loses moisture at a rate that depends on how much wetter it is compared to the moisture level it would naturally reach in balance with the drying air. These models are generally categorized into three groups: theoretical models, which incorporate internal resistance to moisture migration as well as parameters such as diffusivity, thermal conductivity, and material geometry; semi-theoretical models, which are derived from Fick’s 2nd law of diffusion and empirical models, whose parameters are obtained through experimental fitting and therefore lack explicit physical interpretation. Semi-theoretical and empirical models also consider the external resistance to moisture transfer between the product and surrounding air (Chukwunonye, C. D et al. 2016, Onwude, D. I et al. 2016, Mahapatra, A and Tripathy, P.P. 2018). Semi-theoretical models are widely preferred for fruits and vegetables (F&V), with commonly used equations like the Lewis, Page, Henderson and Pabis, Wang and Singh, and Midilli models. Moisture diffusivity in many studies is estimated using Fick’s diffusion law.

Developing mathematical models for carrot drying kinetics and associated quality characteristics is essential, as such information provides valuable guidance for industrial drying operations. Modelling involves formulating equations that closely represent the real drying behaviour, helping in the design, optimisation, and performance evaluation of drying equipment (Celma, A.R et al. 2012). Therefore, the present study investigated the thin-layer drying characteristics of carrot slices using a freeze dryer by evaluating their drying behavior at temperatures ranging from 40 to 60 °C under different slice thickness conditions. Additionally, effective moisture diffusivity and activation energy were determined to better understand the convective drying behaviour of carrot slices.

2. MATERIALS AND METHODS

Carrots of uniform size were washed thoroughly under running tap water to remove any dirt or impurities and then graded to minimise variation in exposed surface area. The cleaned carrots were sliced into thicknesses of 2, 4 and 6 mm using a vegetable slicer equipped with an adjustable cutting blade. The prepared slices were frozen at −40 °C for 24 hours in a deep freezer prior to drying. Freeze drying was carried out in a laboratory-scale freeze dryer (LYODEL-55, Delvac Pumps Pvt. Ltd., Chennai), which consisted of a drying chamber, condenser, vacuum pump, and heating unit. During operation, the vacuum pump removed water vapour from the chamber, which subsequently condensed and formed ice in the condenser. The heating system of the freeze dryer comprised three heating plates mounted on a support stand, as illustrated in Figure. 1.

Figure. 1 Freeze dryer with sliced carrot slices

Heat for the phase change was supplied to the samples through direct contact with the heating plates. Drying was conducted at plate temperatures of 30, 40 and 50 °C, the condenser temperature was −40 °C, and the vacuum pressure was kept below 0.150 mbar. The weight of the samples was measured at 30 (min) intervals, and the process was continued until the equilibrium moisture content (EMC) was reached and a weight change of less than 0.01 g. These weight loss measurements were then used to evaluate the drying behaviour, including moisture content (d.b.) and drying rate (d.b. min⁻¹).

2.1 Modelling of the Thin-Layer Drying

Moisture content of carrot slices was determined by the standard oven method (AOAC, 2000) and was expressed as percent on a dry basis as given in equation (1).

Moisture content % d.b. = W1-W2W2-W×100                     ... equation (1)

where,

W = Weight of empty box, g

W1 = Weight of box with material before drying, g

W2 = Weight of box with dried material, g

The moisture content (MC) (dry basis) of the carrot slices was utilized to calculate the drying rate (DR) under different temperature conditions and slice thicknesses. Drying rate of the agricultural products was calculated as given in equation- 2 (Akpinar, E.K et al. 2006)

DR= Mt +dt-Mtdt                                                          ... equation (2)

where, 𝑀𝑡 = MC at the time, min

Mt+dt = MC at time of t+dt, (kg water/kg dm)

dt = Time interval between 2 points.

Development of suitable drying models is essential for accurately characterized the behaviour of drying carrot slices. In this study, the drying behaviour of carrot slices at different temperatures was analysed and suitable 6 widely used thin-layer drying models, as shown in Table. 1.

Table.1 Different Drying Models Used in Freeze Drying of Carrot Slices

| S. No. | Models | Model | Ref |
| --- | --- | --- | --- |
|  | Page | MR = exp(-ktn) | Page (1949) |
|  | Henderson and Pabis | MR = aexp(-kt) | Westerman (1973) |
|  | Wang and Singh | MR = 1+at +bt2 | Wang and Singh (1978) |
|  | Logarithmic | MR = aexp(-kt)+c | Togrul and Pehlivan (2003) |
|  | Modified Page | MR = exp[(-ktn)] | Overhults et al. (1973) |
|  | Lewis | MR = exp(-kt) | Lewis (1921) |

where

MR = Moisture ratio

k = Drying constant, and a, b, c, n are constants

In the above models, MR represents moisture ratio. The MC of the sample at different stages of drying was normalised by converting into dimensionless form as MR by (Darvishi, H et al. 2012) and can be expressed as given in equation (3):

MR = Mt-Me/ Mi-Me                                                ...equation (3)

where,

MR is the moisture ratio (dimensionless),

Mt is the moisture content at any given time (kg water/ kg solids),

Me is the equilibrium moisture content (kg water/kg solids) and

Mi is the initial moisture content

For most food materials, the equilibrium moisture content (Me) is considerably smaller than both the moisture content at time t (Mt) and the initial moisture content (Mo) (Junling, S et al. 2008). Therefore, Me is often considered negligible, allowing the moisture ratio to be simplified and expressed as shown in equation (4).

MR = MtMo                                             ... equation (4)

In this study, OriginPro 8.0 software (OriginLab, Massachusetts, USA) was used to fit conventional convective drying models to the experimentally obtained moisture ratio (MR) values calculated using Equation 4. Six widely recognised drying models were evaluated to identify the most suitable model for describing the drying characteristics of carrot slices. The performance of each model was assessed based on the R², reduced chi-square (χ²) and RMSE. A better model fit is indicated by higher R² values along with lower χ² and RMSE values (Wang, T et al. 2006, Ertekin, C and Yaldiz, O.S.M.A.N. 2004, Ozkan, I.A et al. 2007).

2.2 Calculation of Effective Diffusivities

In this study, moisture migration during the falling-rate period was assumed to occur primarily through diffusion. According to the movement of moisture from the interior of the product to its surface was reported by Fick’s second law of diffusion, expressed in equation (5).

∂M∂x=Deff ∂2M∂x2                                                      ... equation (5)

The falling-rate drying period in biological materials is commonly described using Fick’s diffusion equation (Wang, T et al. 2006). The analytical solutions presented by Crank (1975) allow this equation to be applied to different regularly shaped products, like rectangular, spherical bodies and cylindrical. For materials with slab geometry, the form of equation (6) is suitable, provided that the initial moisture distribution within the sample is assumed to be uniform.

MRi=8π2n=0∞12n+12exp-(2n+1)2π2Defft4L02⁡                       ... equation (6)

where Deff = Effective diffusivity (m2/s),

L0 = Half thickness of slab (m).

For a longer drying period, equation 6 can be further simplified to only the first term of the series (Tütüncü, M. A & Labuza, T. P. 1996) and can be written in a logarithmic form as follows:

Diffusivities were determined by plotting experimental drying data in terms of ln MR versus drying time in equation (7). The slope and effective diffusivity from equation (6) were calculated as:

lnMR=ln8π2- π2Defft4L02                                      ... equation (7)

2.3 Calculation of Activation Energy

The relationship between activation energy and reaction rate was expressed using the Arrhenius

equation (8):

k = A exp (- Ea / RT)                                             ... equation (8)

The parameter k represents the reaction rate constant, A is a temperature-independent pre-exponential factor, (Ea) activation energy, (R) universal gas constant, and T is the absolute temperature. Since this relationship follows an exponential trend, even minor variations in temperature or activation energy can cause noticeable changes in the reaction rate. In drying studies, the effective moisture diffusivity (Deff) is often treated in a similar way to the reaction rate constant (k). Therefore, its dependence on temperature can also be described using an Arrhenius-type expression, as presented in equation (9).

Deff= Doexp -EaRT                                                    ... equation (9)

where: Do = the pre-exponential factor of the Arrhenius equation in m2/s,

Ea = Activation energy, kJ/mol,

R = Universal gas constant (8.314 kJ/mol K),

T = Absolute air temperature (°K).

Equation by taking the natural logarithm in equation (10).

ln Deff= Do-EaR. 1T                                                               ... equation (10)

The pre-exponential factor and activation energy in the Arrhenius equation were determined by plotting ln Deff against 1/T1, using the experimentally obtained values of effective moisture diffusivity and absolute drying air temperature (Nwajinka, C.O et al. 2014).

Ea is calculated using the slope of the line as

Ea = - (slope x R)                                                      ...equation (11)

3. RESULTS AND DISCUSSION

Carrot slices of different thickness of 2, 4 and 6 mm were used for drying experiments and sliced carrot slices were frozen at −40 °C in a deep freezer. Frozen slices were placed on stainless steel plates and the plates were kept on three heating plates of the freeze dryer at plate temperatures of 40, 50 and 60 °C. Sample weight loss was recorded every 30 minutes until equilibrium moisture content was attained. Heating plate temperature and slice thickness are the important factors influenced the drying characteristics of carrot slices. The variation in MC with drying time at different heating plate temperatures.

3.1 Effect of Slice Thickness on the MC

The influence of slice thickness on MC variation is illustrated in Figure. 2 (a, b, c). An increase in carrot slice thickness resulted in a corresponding increase in drying time. At a heating plate temperature of 60 °C, it required approximately 480 minutes to reduce the MC of 2 mm slices from 809.09% to 8.8% (d.b). Under the same conditions, slices with thicknesses of 4 mm and 6 mm required about 510 minutes and 570 minutes, respectively, to reach a similar final moisture content. A similar trend was observed at heating plate temperatures of 40 °C and 50 °C. These findings are consistent with the observations of (Aremu, M. A & Ekpo, I. C. 2013), who reported that drying time increases with increasing sample thickness.

| (a) | (b) |
| --- | --- |

(c)

### Figure. 2. Effect of carrot slice thickness on the moisture content vs time at heating plate temperature of (a) 40 (b) 50 (c) 60 °C

The drying rate was strongly influenced by slice thickness, as moisture removal became slower with increasing thickness. Among the samples, the 2 mm slices reached equilibrium moisture content the fastest, whereas the 6 mm slices required the longest drying time. This behaviour may be attributed to the shorter moisture migration path in thinner slices, which allows free water to escape more easily from the surface. In contrast, thicker slices present greater internal resistance to moisture movement, thereby slowing the approach to equilibrium moisture content and reducing the overall drying rate (Etoamaihe, U.J and Ibeawuchi, K.O. 2010). As slice thickness increases, the resistance to moisture diffusion also increases, resulting in prolonged drying time (Zarein, M et al. 2013).

3.2 Effect of Slice Thickness on Drying Rate

The drying rate was determined as the reduction in moisture content over the time interval between two successive measurements, divided by the duration of that interval (Onu Chijioke, E et al. 2014). The variation in drying rate with respect to the experimental conditions is illustrated in the figure. 3 (a, b, c). For slices with a thickness of 2 mm, the maximum drying rates are 1.53, 1.30 and 1.14 g/g·min at heating plate temperatures of 60, 50 and 40 °C, respectively. In contrast, the lowest drying rates were recorded for 6 mm slices, with values of 1.18, 1.14 and 1.11 g/g·min at the corresponding temperatures. Overall, the 2 mm slices exhibited higher drying rates than the 4 mm and 6 mm slices. This may be attributed to the shorter moisture migration path and faster evaporation, which facilitate more efficient moisture transfer in thinner slices. It was further observed that drying rates increased with rising drying temperatures. This trend is consistent with earlier findings on thin-layer drying of agricultural and biological materials (Diamante, L.M and Munro, P.A. 1991, Doymaz, I and Pala, M., 2003). Higher temperatures supply greater thermal energy to the product, enhancing the mobility of water molecules and subsequently increasing moisture diffusivity, which leads to faster drying (Junling, S et al. 2008).

| (a) | (b) |
| --- | --- |

(c)

Figure. 3 Effect of carrot slice thickness on drying rate vs time (a) 40 (b) 50 (c) 60 °C

3.3 Moisture Ratio

The moisture ratio decreased progressively as drying advanced, as shown in Figure. 4 (a, b, c) illustrated the variation in moisture ratio with respect to drying time, indicating a continuous decline throughout the process. As expected, increasing the drying air temperature reduced the time required to achieve a given moisture ratio, owing to enhanced heat transfer. This behaviour may be attributed to the greater temperature gradient between the drying air and the product, which promotes faster moisture migration.

| (a) | (b) |
| --- | --- |

(c)

### Figure. 4 Effect of carrot slice thickness on the MC vs time at heating plate temperature (a) 40 °C (b) 50 °C (c) 60 °C

3.4 Effective Moisture Diffusivity and Activation Energy

Drying of food typically takes place in the falling-rate period. The results revealed that moisture removal was mainly by diffusion mechanisms; therefore, Fick’s 2nd law of Diffusion was applied to moisture migration. The findings further indicated that internal mass transfer resistance was the controlling factor for drying time, which consequently resulted in the dominance of the falling-rate period throughout the drying process. By using the curve fitting tools and from equation 11, the effective Deff was estimated in the range of 4.86 × 10-11 to 4.38 × 10-10 to m2/s, which are generally within the range of 10-11 to 10-6 m2/s given for food materials' moisture diffusion coefficient (Wang, Z et al. 2007, Moshen, B. 2016, Babalis, S.J and Belessiotis, V.G. 2004).

Table 2 indicates that the lowest effective moisture diffusivity occurred at the smallest slice thickness, while increasing slice thickness resulted in higher diffusivity values at a constant drying temperature. This observation agrees with the findings of (Aghbashlo, M. and Samimi-Akhijahani, H. 2008). The trend may be attributed to the higher internal moisture gradient developed in thicker samples. Moreover, effective moisture diffusion is influenced by the distance moisture must travel before evaporating to the surroundings, with shorter paths generally enhancing the diffusion process (Afolabi, T.J et al. 2014).

Table. 2 Effect of slice thickness on the effective moisture diffusivity (Deff) and activation energy (Ea)

| S. No. | Slice thickness, mm | Deff, m2/s | Ea, kJ/mol. |
| --- | --- | --- | --- |
|  | 2 | 4.86 × 10-11 | 52.01 |
|  | 4 | 1.94 × 10-10 | 53.82 |
|  | 6 | 4.38 × 10-10 | 57.54 |

The logarithm of Deff was plotted against the reciprocal of absolute temperature, and the plot exhibited a linear trend consistent with the Arrhenius relationship. The Ea was determined from the slope of the line, and the values obtained were 52.01, 53.82, and 57.54 kJ/mol for carrot slices of 2, 4 and 6 mm thickness. Results in Table 2 indicate that activation energy increases with increasing slice thickness, which may be attributed to a longer diffusion path, greater internal moisture gradient, and higher temperature sensitivity. A higher Ea value implies lower resistance of Deff to temperature variation and also reflects the moisture-binding strength within the material (Mahapatra, A. and Tripathy, P.P. 2018). In this study, the activation energy of carrots was found to range between 17.8 and 70.7 kJ/mol, which lies within the general reported range of 12.7 to 110 kJ/mol for food materials (Touil, A et al. 2014).

3.5 Mathematical Models to the Drying Curves

MR was calculated from the moisture content measured during the drying experiments. After fitting the MR variation with drying time to various thin-layer drying models, the corresponding model parameters were found and are shown in Table 1. All of the chosen models had R2 values between 0.98 and 0.99, which shows that the experimental and predicted results were in excellent agreement. For the majority of drying temperatures and slice thicknesses, the logarithmic model offered the best fit. Table 3 summarized the statistical evaluation of all models, included their coefficients and performance metrics like R2 reduced χ² and RMSE. The R2 values were always greater than 0.98, the RMSE and reduced χ² values stayed below 8.81 and 0.019, confirmed a strong agreement between the experimental data and the model predictions.

Table 3. Statistical analysis on the modelling of MC and drying time for the freeze-dried carrot slices

| Model | Drying temperature, °C | Slice thickness, mm | Model constants | R2 | χ2 | RMSE |
| --- | --- | --- | --- | --- | --- | --- |
| Logistic regression model | 40 | 2 | 0.01 | 0.99 | 2.96 | 0.017 |
|  |  | 4 | 0.014 | 0.985 | 7.68 | 0.02 |
|  |  | 6 | 0.014 | 0.98 | 6.37 | 0.02 |
|  | 50 | 2 | 0.01 | 0.99 | 2.80 | 0.01 |
|  |  | 4 | 0.01 | 0.98 | 7.55 | 0.02 |
|  |  | 6 | 0.014 | 0.98 | 6.37 | 0.02 |
|  | 60 | 2 | 0.09 | 0.99 | 4.84 | 0.02 |
|  |  | 4 | 0.008 | 0.99 | 3.57 | 0.018 |
|  |  | 6 | 0.008 | 0.98 | 3.67 | 0.019 |
| Newton model | 40 | 2 | 0.060 | 0.99 | 3.84 | 0.019 |
|  |  | 4 | 0.067 | 0.98 | 8.92 | 0.02 |
|  |  | 6 | 0.04 | 0.99 | 7.86 | 0.02 |
|  | 50 | 2 | 0.070 | 0.98 | 3.64 | 0.01 |
|  |  | 4 | 0.070 | 0.98 | 8.37 | 0.02 |
|  |  | 6 | 0.05 | 0.98 | 8.13 | 0.02 |
|  | 60 | 2 | 0.065 | 0.99 | 5.18 | 0.022 |
|  |  | 4 | 0.057 | 0.99 | 3.86 | 0.019 |
|  |  | 6 | 0.050 | 0.99 | 4.03 | 0.02 |
| Page model | 40 | 2 | 0.06 | 0.99 | 4.05 | 0.020 |
|  |  | 4 | 0.067 | 0.98 | 9.37 | 0.03 |
|  |  | 6 | 0.04 | 0.98 | 8.22 | 0.02 |
|  | 50 | 2 | 0.07 | 0.99 | 3.85 | 0.01 |
|  |  | 4 | 0.07 | 0.98 | 8.81 | 0.02 |
|  |  | 6 | 0.05 | 0.98 | 8.52 | 0.02 |
|  | 60 | 2 | 0.065 | 0.99 | 5.50 | 0.023 |
|  |  | 4 | 0.057 | 0.99 | 4.09 | 0.02 |
|  |  | 6 | 0.05 | 0.99 | 4.24 | 0.02 |
| Modified page model | 40 | 2 | 0.068 | 0.99 | 8.87 | 0.009 |
|  |  | 4 | 0.090 | 0.99 | 8.89 | 0.002 |
|  |  | 6 | 0.049 | 0.98 | 8.22 | 0.004 |
|  | 50 | 2 | 0.080 | 0.99 | 6.39 | 0.008 |
|  |  | 4 | 0.110 | 0.99 | 1.77 | 0.004 |
|  |  | 6 | 0.060 | 0.99 | 1.84 | 0.020 |
|  | 60 | 2 | 0.079 | 0.99 | 2.53 | 0.005 |
|  |  | 4 | 0.064 | 0.99 | 4.02 | 0.006 |
|  |  | 6 | 0.050 | 0.99 | 1.45 | 0.003 |
| Henderson model | 40 | 2 | 0.050 | 0.99 | 4.009 | 0.02 |
|  |  | 4 | 0.060 | 0.99 | 9.3 | 0.03 |
|  |  | 6 | 0.040 | 0.98 | 8.02 | 0.02 |
|  | 50 | 2 | 0.070 | 0.981 | 3.82 | 0.019 |
|  |  | 4 | 0.070 | 0.99 | 8.77 | 0.02 |
|  |  | 6 | 0.050 | 0.99 | 8.36 | 0.02 |
|  | 60 | 2 | 0.060 | 0.99 | 5.43 | 0.02 |
|  |  | 4 | 0.050 | 0.99 | 4.01 | 0.02 |
|  |  | 6 | 0.040 | 0.98 | 4.11 | 0.02 |

4. CONCLUSION

Accurate information for assessing the thin-layer drying behaviour of carrot slices under freeze-drying conditions were obtained from the drying experiments. Several semi-theoretical and empirical drying models were fitted to the experimental data, and their predictive performance was assessed using statistical indicators such as R², RMSE and reduced chi-square. Overall, most of the models exhibited reliable predictive capability according to these criteria. However, the logarithmic model showed superior performance, consistently yielding higher R² values along with lower RMSE and chi-square values compared to the other models. Consequently, the logarithmic model is thought to be the best model for precisely characterizing the drying kinetics and moisture removal properties of carrot slices.

5. REFERENCES

- Beaudry, C., Raghavan, G. S. V., & Rennie, T. J. (2003). Microwave finish drying of osmotically dehydrated cranberries. Drying Technology, 21(9), 1797-1810.

- Ahmad, T., Cawood, M., Iqbal, Q., Ariño, A., Batool, A., Tariq, R. M. S., ... & Akhtar, S. (2019). Phytochemicals in Daucus carota and their health benefits. Foods, 8(9), 424.

- Ozkan, I. A., Akbudak, B., & Akbudak, N. (2007). Microwave drying characteristics of spinach. Journal of food engineering, 78(2), 577-583.

- Al-Harahsheh, M., Ala’a, H., & Magee, T. R. A. (2009). Microwave drying kinetics of tomato pomace: Effect of osmotic dehydration. Chemical Engineering and Processing: Process Intensification, 48(1), 524-531.

- Soysal, Y., Öztekin, S., & Eren, Ö. (2006). Microwave drying of parsley: modelling, kinetics, and energy aspects. Biosystems Engineering, 93(4), 403-413.

- Chukwunonye, C. D., Nnaemeka, N. R., Chijioke, O. V., & Obiora, N. C. (2016). Thin layer drying modelling for some selected Nigerian produce: a review. Am J Food Sci Nutr Res, 3(1), 1-15.

- Onwude, D. I., Hashim, N., Janius, R. B., Nawi, N., & Abdan, K. (2016). Modelling the convective drying process of pumpkin (Cucurbita moschata) using an artificial neural network. International food research journal, 23, S237.

- Akpinar, E., Midilli, A. D. N. A. N., & Bicer, Y. (2003). Single layer drying behaviour of potato slices in a convective cyclone dryer and mathematical modeling. Energy conversion and management, 44(10), 1689-1705.

- Chakraverty, A., Mujumdar, A.S. and Ramaswamy, H.S. eds., 2003. Handbook of postharvest technology: cereals, fruits, vegetables, tea, and spices (Vol. 93). CRC press.

- Mahapatra, A. and Tripathy, P.P., 2018. Modeling and simulation of moisture transfer during solar drying of carrot slices. Journal of Food Process Engineering, 41(8), p.e12909.

- Celma, A. R., Cuadros, F., & López-Rodríguez, F. (2012). Convective drying characteristics of sludge from treatment plants in tomato processing industries. Food and Bioproducts Processing, 90(2), 224-234.

- Association of Official Analytical Chemists, 2000. Official methods of analysis of the Association of Official Analytical Chemists (Vol. 11). The Association.

- Akpinar, E. K., Bicer, Y., & Cetinkaya, F. (2006). Modelling of thin layer drying of parsley leaves in a convective dryer and under open sun. Journal of food engineering, 75(3), 308-315.

- Darvishi, H., Banakar, A., & Zarein, M. (2012). Mathematical modeling and thin layer drying kinetics of carrot slices. Global Journal of Science Frontier Research Mathematics and Decision Sciences, 12(7), 56-64.

- Junling, S., Zhongli, P., Tara, H. M., Delilah,W., Edward, H. and Don, O. (2008) Elsevier – Food Science and Technology. 41, 1962 – 1972.

- Wang, T., Deng, Y., Geng, Y., Gao, Z., Zou, J., & Wang, Z. (2006). Preparation of submicron unilamellar liposomes by freeze-drying double emulsions. Biochimica et biophysica acta (bba)-biomembranes, 1758(2), 222-231.

- Ertekin, C., & Yaldiz, O. S. M. A. N. (2004). Drying of eggplant and selection of a suitable thin layer drying model. Journal of food engineering, 63(3), 349-359.

- Ozkan, I. A., Akbudak, B., & Akbudak, N. (2007). Microwave drying characteristics of spinach. Journal of food engineering, 78(2), 577-583.

- Tütüncü, M. A., & Labuza, T. P. (1996). Effect of geometry on the effective moisture transfer diffusion coefficient. Journal of Food Engineering, 30(3-4), 433-447.

- Nwajinka, C. O., Nwuba, E. I. U., & Udoye, B. O. (2014). Moisture diffusivity and activation energy of drying of melon seeds. Int. J. App. Sci. Eng.-2014.–2 (2).–P, 37-43.

- Aremu, M. A., & Ekpo, I. C. (2013). Determinants of capital structure in Nigerian banking sector. International Journal of Academic Reserach in Economics and Management Sciences, 2(4).

- Etoamaihe, U. J., & Ibeawuchi, K. O. (2010). Prediction of the drying rates of cassava slices during oven drying. Journal of Eng and Applies sciences, 5(4), 308-311.

- Zarein, M., Samadi, S. H., & Ghobadian, B. (2013). Kinetic drying and mathematical modeling of apple slices on dehydration process.

- Onu Chijioke, E., Igbokwe Philomena, K., & Nwabanne Joseph, T. (2017). Effective moisture diffusivity, activation energy and specific energy consumption in the thin-layer drying of potato. Int. J. Nov. Res. Eng. Sci, 3(2), 10-22.

- Diamante, L. M., & Munro, P. A. (1991). Mathematical modelling of hot air drying of sweet potato slices. International journal of food science & technology, 26(1), 99-109.

- Doymaz, İ., & Pala, M. (2003). The thin-layer drying characteristics of corn. Journal of food engineering, 60(2), 125-130.

- Wang, Z., Sun, J., Liao, X., Chen, F., Zhao, G., Wu, J., & Hu, X. (2007). Mathematical modeling on hot air drying of thin layer apple pomace. Food Research International, 40(1), 39-46.

- Moshen, B. (2016) Energy efficiency and moisture diffusivity of apple slices during convective drying. Food science and technology. 36, (1) :145-150.

- Babalis, S. J., & Belessiotis, V. G. (2004). Influence of the drying conditions on the drying constants and moisture diffusivity during the thin-layer drying of figs. Journal of food Engineering, 65(3), 449-458.

- Aghbashlo, M., & Samimi-Akhijahani, H. (2008). Influence of drying conditions on the effective moisture diffusivity, energy of activation and energy consumption during the thin-layer drying of berberis fruit (Berberidaceae). Energy Conversion and Management, 49(10), 2865-2871.

- Afolabi, T. J., Tunde-Akintunde, T. Y., & Oyelade, O. J. (2014). Influence of drying conditions on the effective moisture diffusivity and energy requirements of ginger slices. Journal of Food Research, 3(5), 103.

- Touil, A., Chemkhi, S., & Zagrouba, F. (2014). Moisture diffusivity and shrinkage of fruit and cladode of Opuntia ficus-indica during infrared drying. Journal of food processing, 2014(1), 175402.