# Advancements in Irrigation Technologies and Water Management Strategies for Sustainable Agriculture: A Comprehensive Review

# Abstract

Sustainable agriculture faces escalating pressures from climate change, population growth, and finite natural resources, with the sector currently accounting for approximately 70 % of global freshwater withdrawals. The present review examines the critical shift from traditional, gravity-fed irrigation methods—such as flooding and furrow systems—which are often inefficient due to excessive evaporation and runoff losses  apart from  non uniform distribution. The traditional practices not only deplete water resources but also demand high energy and contribute to soil degradation  and reduced crop productivity.

The review highlights modern advancements in irrigation technologies, including drip, micro, sprinkler, and subsurface systems, which dramatically enhance water use efficiency. Specifically, drip and micro-irrigation systems achieve application efficiencies of 90–95% by delivering water directly to plant root zones through integration of precision agriculture tools of sensors, remote sensing technique, IoT, and AI—enables real-time monitoring and adaptive scheduling to optimize water delivery.

In addition to technological innovations, the review evaluates broader water harvesting strategies such as rainwater harvesting, wastewater recycling, desalination, and deficit irrigation. It also discusses the role of genetic advancements in developing drought-resistant crops to enhance resilience against climate variability. By shifting toward these precision and sustainable alternatives, the agricultural sector can mitigate environmental degradation, reduce its carbon footprint, and secure global food security. Widespread adoption of these advanced technologies and strategies is presented as an essential pathway for the long-term viability of agriculture in a resource-constrained regions.

Keywords: Precision irrigation, Water use efficiency (WUE), Sustainable agriculture, IoT and AI in agriculture, Climate change resilience, Modern water management.

## Introduction

The agricultural sector, an integral part of global food security and economic stability, confronts escalating pressures from climate change, burgeoning populations, and finite natural resources, accounting for approximately 70% of global freshwater withdrawals and ranking as one of the largest energy consumers in many regions (Daraz et al., 2025)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlVtYXIiLCJmYW1pbHkiOiJEYXJheiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiLFoHRlZmFuIiwiZmFtaWx5IjoiQm9qbmVjIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllvdW5hcyIsImZhbWlseSI6IktoYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMDc3LTA0NzIiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzOTAvYWdyaWN1bHR1cmUxNTA1MDU1NCIsIm9wZW5hbGV4IjoiVzQ0MDgyMTk4MTEiLCJleGEiOiJodHRwczovL3d3dy5tZHBpLmNvbS8yMDc3LTA0NzIvMTUvNS81NTQifSwidGl0bGUiOiJFbmVyZ3ktRWZmaWNpZW50IFNtYXJ0IElycmlnYXRpb24gVGVjaG5vbG9naWVzOiBBIFBhdGh3YXkgdG8gV2F0ZXIgYW5kIEVuZXJneSBTdXN0YWluYWJpbGl0eSBpbiBBZ3JpY3VsdHVyZSIsImFic3RyYWN0IjoiVGhlIGFncmljdWx0dXJhbCBzZWN0b3IgZmFjZXMgY2hhbGxlbmdlcyBzdWNoIGFzIHdhdGVyIHNjYXJjaXR5LCBlbmVyZ3kgaW5lZmZpY2llbmN5LCBhbmQgZGVjbGluaW5nIHByb2R1Y3Rpdml0eSwgcGFydGljdWxhcmx5IGluIGFyaWQgcmVnaW9ucy4gVHJhZGl0aW9uYWwgaXJyaWdhdGlvbiBtZXRob2RzIGNvbnRyaWJ1dGUgdG8gcmVzb3VyY2UgZGVwbGV0aW9uIGFuZCBlbnZpcm9ubWVudGFsIGltcGFjdHMuIFNvbGFyLXBvd2VyZWQgc21hcnQgaXJyaWdhdGlvbiBzeXN0ZW1zIGludGVncmF0ZSBwcmVjaXNpb24gaXJyaWdhdGlvbiB3aXRoIHJlbmV3YWJsZSBlbmVyZ3ksIGltcHJvdmluZyB3YXRlciB1c2UgYW5kIHByb2R1Y3Rpdml0eS4gSW4gUGFraXN0YW4sIHdoZXJlIGFncmljdWx0dXJlIGNvbnRyaWJ1dGVzIDE5JSBvZiBncm9zcyBkb21lc3RpYyBwcm9kdWN0IGFuZCBlbXBsb3lzIDQwJSBvZiB0aGUgd29ya2ZvcmNlLCB0aGVzZSBjaGFsbGVuZ2VzIGFyZSBzZXZlcmUsIGVzcGVjaWFsbHkgaW4gd2F0ZXItc2NhcmNlIGFyZWFzIGxpa2UgdGhlIENob2xpc3RhbiBEZXNlcnQuIFRoaXMgc3R1ZHkgZXhhbWluZXMgdGhlIGltcGFjdCBvZiBzb2xhci1wb3dlcmVkIHNtYXJ0IGlycmlnYXRpb24gb24gYWdyaWN1bHR1cmFsIHByb2R1Y3Rpdml0eSwgd2F0ZXIgY29uc2VydmF0aW9uLCBhbmQgZW5lcmd5IGVmZmljaWVuY3kgaW4gdGhlIENob2xpc3RhbiBEZXNlcnQuIFVzaW5nIGEgcXVhbnRpdGF0aXZlIGNyb3NzLXNlY3Rpb25hbCBkZXNpZ24sIGRhdGEgd2VyZSBjb2xsZWN0ZWQgZnJvbSAzODQgZmFybWVycyB2aWEgc3RydWN0dXJlZCBxdWVzdGlvbm5haXJlcy4gU3RhdGlzdGljYWwgYW5hbHlzZXMsIGluY2x1ZGluZyBtdWx0aXBsZSBsaW5lYXIgcmVncmVzc2lvbiwgcGFpcmVkIHNhbXBsZSB0LXRlc3RzLCBhbmQgU3RydWN0dXJhbCBFcXVhdGlvbiBNb2RlbGluZyAoU0VNKSwgd2VyZSBjb25kdWN0ZWQuIEZpbmRpbmdzIHNob3cgc2lnbmlmaWNhbnQgaW1wcm92ZW1lbnRzIGluIGNyb3AgeWllbGQgKGZyb20gMy4wIHRvIDQuOCB0b25zL2hlY3RhcmUpIGFuZCByZWR1Y3Rpb25zIGluIHdhdGVyIGFuZCBlbmVyZ3kgY29uc3VtcHRpb24uIFJlZ3Jlc3Npb24gYW5hbHlzaXMgaGlnaGxpZ2h0ZWQgc3Ryb25nIHBvc2l0aXZlIGVmZmVjdHMgb24geWllbGQgYW5kIGVmZmljaWVuY3ksIHdoaWxlIFNFTSBjb25maXJtZWQgcmVkdWNlZCBlbnZpcm9ubWVudGFsIGltcGFjdCBhbmQgb3BlcmF0aW9uYWwgY29zdHMuIFRoZSBzdHVkeSBjb25jbHVkZXMgdGhhdCBzb2xhci1wb3dlcmVkIGlycmlnYXRpb24gZW5oYW5jZXMgcHJvZHVjdGl2aXR5LCBjb25zZXJ2ZXMgcmVzb3VyY2VzLCBhbmQgcHJvbW90ZXMgc3VzdGFpbmFiaWxpdHkuIFBvbGljeW1ha2VycyBzaG91bGQgcHJvdmlkZSBmaW5hbmNpYWwgaW5jZW50aXZlcywgaW52ZXN0IGluIHJlbmV3YWJsZSBpbmZyYXN0cnVjdHVyZSwgYW5kIGltcGxlbWVudCB0cmFpbmluZyBwcm9ncmFtcyB0byBzdXBwb3J0IGFkb3B0aW9uLiBDb2xsYWJvcmF0aXZlIGVmZm9ydHMgYXJlIGVzc2VudGlhbCBmb3Igc3VzdGFpbmFibGUgYWdyaWN1bHR1cmUgaW4gYXJpZCByZWdpb25zLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJpc3N1ZWREYXRlIjoiMjAyNS0wMy0wNSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkFncmljdWx0dXJlIiwiaXNzdWUiOjUsInZvbHVtZSI6MTUsImltcGFjdEZhY3RvciI6IjUuMjYifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjo1NTQsImxhc3RQYWdlIjo1NTR9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiTXVsdGlkaXNjaXBsaW5hcnkgRGlnaXRhbCBQdWJsaXNoaW5nIEluc3RpdHV0ZSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NDgsImNpdGVkQnlDb3VudCI6Mn19LCJxdW90ZSI6IjEuIEludHJvZHVjdGlvbiBUaGUgYWdyaWN1bHR1cmFsIHNlY3RvciwgYSBjb3JuZXJzdG9uZSBvZiBnbG9iYWwgZm9vZCBzZWN1cml0eSBhbmQgZWNvbm9taWMgZGV2ZWxvcG1lbnQsIGZhY2VzIG1vdW50aW5nIGNoYWxsZW5nZXMgaW4gdGhlIDIxc3QgY2VudHVyeS4gQ2xpbWF0ZSBjaGFuZ2UsIHJhcGlkIHBvcHVsYXRpb24gZ3Jvd3RoLCBhbmQgZHdpbmRsaW5nIG5hdHVyYWwgcmVzb3VyY2VzIGhhdmUgaW50ZW5zaWZpZWQgdGhlIG5lZWQgZm9yIHN1c3RhaW5hYmxlIGFncmljdWx0dXJhbCBwcmFjdGljZXMuIEFtb25nIHRoZXNlIGNoYWxsZW5nZXMsIHdhdGVyIHNjYXJjaXR5IGFuZCBlbmVyZ3kgaW5lZmZpY2llbmN5IHN0YW5kIG91dCBhcyBjcml0aWNhbCBiYXJyaWVycyB0byBwcm9kdWN0aXZpdHkgYW5kIHN1c3RhaW5hYmlsaXR5LiBHbG9iYWxseSwgYWdyaWN1bHR1cmUgYWNjb3VudHMgZm9yIGFwcHJveGltYXRlbHkgNzAlIG9mIGZyZXNod2F0ZXIgd2l0aGRyYXdhbHMgYW5kIGlzIG9uZSBvZiB0aGUgbGFyZ2VzdCBjb25zdW1lcnMgb2YgZW5lcmd5IGluIG1hbnkgcmVnaW9ucyBbIDEsIDJdLiBUcmFkaXRpb25hbCBpcnJpZ2F0aW9uIHByYWN0aWNlcywgb2Z0ZW4gaW5lZmZpY2llbnQgYW5kIHJlc291cmNlLWludGVuc2l2ZSwgZXhhY2VyYmF0ZSB0aGVzZSBpc3N1ZXMsIGxlYWRpbmcgdG8gd2F0ZXIgd2FzdGFnZSwgZW5lcmd5IG92ZXJ1c2UsIGFuZCByZWR1Y2VkIGNyb3AgeWllbGRzLiJ9XQ%3D%3D. These challenges are intensified by water scarcity and energy inefficiency, which undermine productivity and sustainability. Traditional irrigation methods, characterized by huge conveyance losses, contribute significantly to loss of water and consume more energy and diminished crop yields (Daraz et al., 2025)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlVtYXIiLCJmYW1pbHkiOiJEYXJheiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiLFoHRlZmFuIiwiZmFtaWx5IjoiQm9qbmVjIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllvdW5hcyIsImZhbWlseSI6IktoYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMDc3LTA0NzIiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzOTAvYWdyaWN1bHR1cmUxNTA1MDU1NCIsIm9wZW5hbGV4IjoiVzQ0MDgyMTk4MTEiLCJleGEiOiJodHRwczovL3d3dy5tZHBpLmNvbS8yMDc3LTA0NzIvMTUvNS81NTQifSwidGl0bGUiOiJFbmVyZ3ktRWZmaWNpZW50IFNtYXJ0IElycmlnYXRpb24gVGVjaG5vbG9naWVzOiBBIFBhdGh3YXkgdG8gV2F0ZXIgYW5kIEVuZXJneSBTdXN0YWluYWJpbGl0eSBpbiBBZ3JpY3VsdHVyZSIsImFic3RyYWN0IjoiVGhlIGFncmljdWx0dXJhbCBzZWN0b3IgZmFjZXMgY2hhbGxlbmdlcyBzdWNoIGFzIHdhdGVyIHNjYXJjaXR5LCBlbmVyZ3kgaW5lZmZpY2llbmN5LCBhbmQgZGVjbGluaW5nIHByb2R1Y3Rpdml0eSwgcGFydGljdWxhcmx5IGluIGFyaWQgcmVnaW9ucy4gVHJhZGl0aW9uYWwgaXJyaWdhdGlvbiBtZXRob2RzIGNvbnRyaWJ1dGUgdG8gcmVzb3VyY2UgZGVwbGV0aW9uIGFuZCBlbnZpcm9ubWVudGFsIGltcGFjdHMuIFNvbGFyLXBvd2VyZWQgc21hcnQgaXJyaWdhdGlvbiBzeXN0ZW1zIGludGVncmF0ZSBwcmVjaXNpb24gaXJyaWdhdGlvbiB3aXRoIHJlbmV3YWJsZSBlbmVyZ3ksIGltcHJvdmluZyB3YXRlciB1c2UgYW5kIHByb2R1Y3Rpdml0eS4gSW4gUGFraXN0YW4sIHdoZXJlIGFncmljdWx0dXJlIGNvbnRyaWJ1dGVzIDE5JSBvZiBncm9zcyBkb21lc3RpYyBwcm9kdWN0IGFuZCBlbXBsb3lzIDQwJSBvZiB0aGUgd29ya2ZvcmNlLCB0aGVzZSBjaGFsbGVuZ2VzIGFyZSBzZXZlcmUsIGVzcGVjaWFsbHkgaW4gd2F0ZXItc2NhcmNlIGFyZWFzIGxpa2UgdGhlIENob2xpc3RhbiBEZXNlcnQuIFRoaXMgc3R1ZHkgZXhhbWluZXMgdGhlIGltcGFjdCBvZiBzb2xhci1wb3dlcmVkIHNtYXJ0IGlycmlnYXRpb24gb24gYWdyaWN1bHR1cmFsIHByb2R1Y3Rpdml0eSwgd2F0ZXIgY29uc2VydmF0aW9uLCBhbmQgZW5lcmd5IGVmZmljaWVuY3kgaW4gdGhlIENob2xpc3RhbiBEZXNlcnQuIFVzaW5nIGEgcXVhbnRpdGF0aXZlIGNyb3NzLXNlY3Rpb25hbCBkZXNpZ24sIGRhdGEgd2VyZSBjb2xsZWN0ZWQgZnJvbSAzODQgZmFybWVycyB2aWEgc3RydWN0dXJlZCBxdWVzdGlvbm5haXJlcy4gU3RhdGlzdGljYWwgYW5hbHlzZXMsIGluY2x1ZGluZyBtdWx0aXBsZSBsaW5lYXIgcmVncmVzc2lvbiwgcGFpcmVkIHNhbXBsZSB0LXRlc3RzLCBhbmQgU3RydWN0dXJhbCBFcXVhdGlvbiBNb2RlbGluZyAoU0VNKSwgd2VyZSBjb25kdWN0ZWQuIEZpbmRpbmdzIHNob3cgc2lnbmlmaWNhbnQgaW1wcm92ZW1lbnRzIGluIGNyb3AgeWllbGQgKGZyb20gMy4wIHRvIDQuOCB0b25zL2hlY3RhcmUpIGFuZCByZWR1Y3Rpb25zIGluIHdhdGVyIGFuZCBlbmVyZ3kgY29uc3VtcHRpb24uIFJlZ3Jlc3Npb24gYW5hbHlzaXMgaGlnaGxpZ2h0ZWQgc3Ryb25nIHBvc2l0aXZlIGVmZmVjdHMgb24geWllbGQgYW5kIGVmZmljaWVuY3ksIHdoaWxlIFNFTSBjb25maXJtZWQgcmVkdWNlZCBlbnZpcm9ubWVudGFsIGltcGFjdCBhbmQgb3BlcmF0aW9uYWwgY29zdHMuIFRoZSBzdHVkeSBjb25jbHVkZXMgdGhhdCBzb2xhci1wb3dlcmVkIGlycmlnYXRpb24gZW5oYW5jZXMgcHJvZHVjdGl2aXR5LCBjb25zZXJ2ZXMgcmVzb3VyY2VzLCBhbmQgcHJvbW90ZXMgc3VzdGFpbmFiaWxpdHkuIFBvbGljeW1ha2VycyBzaG91bGQgcHJvdmlkZSBmaW5hbmNpYWwgaW5jZW50aXZlcywgaW52ZXN0IGluIHJlbmV3YWJsZSBpbmZyYXN0cnVjdHVyZSwgYW5kIGltcGxlbWVudCB0cmFpbmluZyBwcm9ncmFtcyB0byBzdXBwb3J0IGFkb3B0aW9uLiBDb2xsYWJvcmF0aXZlIGVmZm9ydHMgYXJlIGVzc2VudGlhbCBmb3Igc3VzdGFpbmFibGUgYWdyaWN1bHR1cmUgaW4gYXJpZCByZWdpb25zLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJpc3N1ZWREYXRlIjoiMjAyNS0wMy0wNSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkFncmljdWx0dXJlIiwiaXNzdWUiOjUsInZvbHVtZSI6MTUsImltcGFjdEZhY3RvciI6IjUuMjYifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjo1NTQsImxhc3RQYWdlIjo1NTR9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiTXVsdGlkaXNjaXBsaW5hcnkgRGlnaXRhbCBQdWJsaXNoaW5nIEluc3RpdHV0ZSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NDgsImNpdGVkQnlDb3VudCI6Mn19LCJxdW90ZSI6IjEuIEludHJvZHVjdGlvbiBUaGUgYWdyaWN1bHR1cmFsIHNlY3RvciwgYSBjb3JuZXJzdG9uZSBvZiBnbG9iYWwgZm9vZCBzZWN1cml0eSBhbmQgZWNvbm9taWMgZGV2ZWxvcG1lbnQsIGZhY2VzIG1vdW50aW5nIGNoYWxsZW5nZXMgaW4gdGhlIDIxc3QgY2VudHVyeS4gQ2xpbWF0ZSBjaGFuZ2UsIHJhcGlkIHBvcHVsYXRpb24gZ3Jvd3RoLCBhbmQgZHdpbmRsaW5nIG5hdHVyYWwgcmVzb3VyY2VzIGhhdmUgaW50ZW5zaWZpZWQgdGhlIG5lZWQgZm9yIHN1c3RhaW5hYmxlIGFncmljdWx0dXJhbCBwcmFjdGljZXMuIEFtb25nIHRoZXNlIGNoYWxsZW5nZXMsIHdhdGVyIHNjYXJjaXR5IGFuZCBlbmVyZ3kgaW5lZmZpY2llbmN5IHN0YW5kIG91dCBhcyBjcml0aWNhbCBiYXJyaWVycyB0byBwcm9kdWN0aXZpdHkgYW5kIHN1c3RhaW5hYmlsaXR5LiBHbG9iYWxseSwgYWdyaWN1bHR1cmUgYWNjb3VudHMgZm9yIGFwcHJveGltYXRlbHkgNzAlIG9mIGZyZXNod2F0ZXIgd2l0aGRyYXdhbHMgYW5kIGlzIG9uZSBvZiB0aGUgbGFyZ2VzdCBjb25zdW1lcnMgb2YgZW5lcmd5IGluIG1hbnkgcmVnaW9ucyBbIDEsIDJdLiBUcmFkaXRpb25hbCBpcnJpZ2F0aW9uIHByYWN0aWNlcywgb2Z0ZW4gaW5lZmZpY2llbnQgYW5kIHJlc291cmNlLWludGVuc2l2ZSwgZXhhY2VyYmF0ZSB0aGVzZSBpc3N1ZXMsIGxlYWRpbmcgdG8gd2F0ZXIgd2FzdGFnZSwgZW5lcmd5IG92ZXJ1c2UsIGFuZCByZWR1Y2VkIGNyb3AgeWllbGRzLiJ9XQ%3D%3D.

### Table 1: Evolution of Irrigation Systems

| Era | Technique | Primary Mechanism | Key Characteristics |
| --- | --- | --- | --- |
| Ancient | Canal & Ditch | Gravity/Manual labor | Rudimentary river water diversion |
| Ancient | Basin Irrigation | Seasonal floods | Rely on seasonal river flooding |
| Traditional | Flood Irrigation | Surface gravity | High water wastage and runoff |
| Traditional | Furrow Irrigation | Small channels | Inefficient gravity-fed distribution |
| Traditional | Border Irrigation | Guided strips | Prone to nonuniform distribution |
| Modern | Drip/Micro | Low-pressure emitters | Targets root zones directly |
| Modern | Sprinkler Systems | Pressurized spray | Uniform field-wide distribution |
| Modern | Subsurface Drip | Buried delivery | Minimizes surface evaporation and lateral damage |

In response, advancements in irrigation technologies and water management strategies have emerged as critical solutions for sustainable agriculture. Modern systems such as drip, sprinkler, and subsurface irrigation optimize water delivery, minimizing evaporation and runoff. Precision agriculture integrates sensors, remote sensing, IoT, and AI to enable real-time monitoring and adaptive scheduling, enhancing water use efficiency (Daraz et al., 2025)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlVtYXIiLCJmYW1pbHkiOiJEYXJheiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiLFoHRlZmFuIiwiZmFtaWx5IjoiQm9qbmVjIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllvdW5hcyIsImZhbWlseSI6IktoYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMDc3LTA0NzIiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzOTAvYWdyaWN1bHR1cmUxNTA1MDU1NCIsIm9wZW5hbGV4IjoiVzQ0MDgyMTk4MTEiLCJleGEiOiJodHRwczovL3d3dy5tZHBpLmNvbS8yMDc3LTA0NzIvMTUvNS81NTQifSwidGl0bGUiOiJFbmVyZ3ktRWZmaWNpZW50IFNtYXJ0IElycmlnYXRpb24gVGVjaG5vbG9naWVzOiBBIFBhdGh3YXkgdG8gV2F0ZXIgYW5kIEVuZXJneSBTdXN0YWluYWJpbGl0eSBpbiBBZ3JpY3VsdHVyZSIsImFic3RyYWN0IjoiVGhlIGFncmljdWx0dXJhbCBzZWN0b3IgZmFjZXMgY2hhbGxlbmdlcyBzdWNoIGFzIHdhdGVyIHNjYXJjaXR5LCBlbmVyZ3kgaW5lZmZpY2llbmN5LCBhbmQgZGVjbGluaW5nIHByb2R1Y3Rpdml0eSwgcGFydGljdWxhcmx5IGluIGFyaWQgcmVnaW9ucy4gVHJhZGl0aW9uYWwgaXJyaWdhdGlvbiBtZXRob2RzIGNvbnRyaWJ1dGUgdG8gcmVzb3VyY2UgZGVwbGV0aW9uIGFuZCBlbnZpcm9ubWVudGFsIGltcGFjdHMuIFNvbGFyLXBvd2VyZWQgc21hcnQgaXJyaWdhdGlvbiBzeXN0ZW1zIGludGVncmF0ZSBwcmVjaXNpb24gaXJyaWdhdGlvbiB3aXRoIHJlbmV3YWJsZSBlbmVyZ3ksIGltcHJvdmluZyB3YXRlciB1c2UgYW5kIHByb2R1Y3Rpdml0eS4gSW4gUGFraXN0YW4sIHdoZXJlIGFncmljdWx0dXJlIGNvbnRyaWJ1dGVzIDE5JSBvZiBncm9zcyBkb21lc3RpYyBwcm9kdWN0IGFuZCBlbXBsb3lzIDQwJSBvZiB0aGUgd29ya2ZvcmNlLCB0aGVzZSBjaGFsbGVuZ2VzIGFyZSBzZXZlcmUsIGVzcGVjaWFsbHkgaW4gd2F0ZXItc2NhcmNlIGFyZWFzIGxpa2UgdGhlIENob2xpc3RhbiBEZXNlcnQuIFRoaXMgc3R1ZHkgZXhhbWluZXMgdGhlIGltcGFjdCBvZiBzb2xhci1wb3dlcmVkIHNtYXJ0IGlycmlnYXRpb24gb24gYWdyaWN1bHR1cmFsIHByb2R1Y3Rpdml0eSwgd2F0ZXIgY29uc2VydmF0aW9uLCBhbmQgZW5lcmd5IGVmZmljaWVuY3kgaW4gdGhlIENob2xpc3RhbiBEZXNlcnQuIFVzaW5nIGEgcXVhbnRpdGF0aXZlIGNyb3NzLXNlY3Rpb25hbCBkZXNpZ24sIGRhdGEgd2VyZSBjb2xsZWN0ZWQgZnJvbSAzODQgZmFybWVycyB2aWEgc3RydWN0dXJlZCBxdWVzdGlvbm5haXJlcy4gU3RhdGlzdGljYWwgYW5hbHlzZXMsIGluY2x1ZGluZyBtdWx0aXBsZSBsaW5lYXIgcmVncmVzc2lvbiwgcGFpcmVkIHNhbXBsZSB0LXRlc3RzLCBhbmQgU3RydWN0dXJhbCBFcXVhdGlvbiBNb2RlbGluZyAoU0VNKSwgd2VyZSBjb25kdWN0ZWQuIEZpbmRpbmdzIHNob3cgc2lnbmlmaWNhbnQgaW1wcm92ZW1lbnRzIGluIGNyb3AgeWllbGQgKGZyb20gMy4wIHRvIDQuOCB0b25zL2hlY3RhcmUpIGFuZCByZWR1Y3Rpb25zIGluIHdhdGVyIGFuZCBlbmVyZ3kgY29uc3VtcHRpb24uIFJlZ3Jlc3Npb24gYW5hbHlzaXMgaGlnaGxpZ2h0ZWQgc3Ryb25nIHBvc2l0aXZlIGVmZmVjdHMgb24geWllbGQgYW5kIGVmZmljaWVuY3ksIHdoaWxlIFNFTSBjb25maXJtZWQgcmVkdWNlZCBlbnZpcm9ubWVudGFsIGltcGFjdCBhbmQgb3BlcmF0aW9uYWwgY29zdHMuIFRoZSBzdHVkeSBjb25jbHVkZXMgdGhhdCBzb2xhci1wb3dlcmVkIGlycmlnYXRpb24gZW5oYW5jZXMgcHJvZHVjdGl2aXR5LCBjb25zZXJ2ZXMgcmVzb3VyY2VzLCBhbmQgcHJvbW90ZXMgc3VzdGFpbmFiaWxpdHkuIFBvbGljeW1ha2VycyBzaG91bGQgcHJvdmlkZSBmaW5hbmNpYWwgaW5jZW50aXZlcywgaW52ZXN0IGluIHJlbmV3YWJsZSBpbmZyYXN0cnVjdHVyZSwgYW5kIGltcGxlbWVudCB0cmFpbmluZyBwcm9ncmFtcyB0byBzdXBwb3J0IGFkb3B0aW9uLiBDb2xsYWJvcmF0aXZlIGVmZm9ydHMgYXJlIGVzc2VudGlhbCBmb3Igc3VzdGFpbmFibGUgYWdyaWN1bHR1cmUgaW4gYXJpZCByZWdpb25zLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJpc3N1ZWREYXRlIjoiMjAyNS0wMy0wNSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkFncmljdWx0dXJlIiwiaXNzdWUiOjUsInZvbHVtZSI6MTUsImltcGFjdEZhY3RvciI6IjUuMjYifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjo1NTQsImxhc3RQYWdlIjo1NTR9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiTXVsdGlkaXNjaXBsaW5hcnkgRGlnaXRhbCBQdWJsaXNoaW5nIEluc3RpdHV0ZSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NDgsImNpdGVkQnlDb3VudCI6Mn19LCJxdW90ZSI6IjEuIEludHJvZHVjdGlvbiBUaGUgYWdyaWN1bHR1cmFsIHNlY3RvciwgYSBjb3JuZXJzdG9uZSBvZiBnbG9iYWwgZm9vZCBzZWN1cml0eSBhbmQgZWNvbm9taWMgZGV2ZWxvcG1lbnQsIGZhY2VzIG1vdW50aW5nIGNoYWxsZW5nZXMgaW4gdGhlIDIxc3QgY2VudHVyeS4gQ2xpbWF0ZSBjaGFuZ2UsIHJhcGlkIHBvcHVsYXRpb24gZ3Jvd3RoLCBhbmQgZHdpbmRsaW5nIG5hdHVyYWwgcmVzb3VyY2VzIGhhdmUgaW50ZW5zaWZpZWQgdGhlIG5lZWQgZm9yIHN1c3RhaW5hYmxlIGFncmljdWx0dXJhbCBwcmFjdGljZXMuIEFtb25nIHRoZXNlIGNoYWxsZW5nZXMsIHdhdGVyIHNjYXJjaXR5IGFuZCBlbmVyZ3kgaW5lZmZpY2llbmN5IHN0YW5kIG91dCBhcyBjcml0aWNhbCBiYXJyaWVycyB0byBwcm9kdWN0aXZpdHkgYW5kIHN1c3RhaW5hYmlsaXR5LiBHbG9iYWxseSwgYWdyaWN1bHR1cmUgYWNjb3VudHMgZm9yIGFwcHJveGltYXRlbHkgNzAlIG9mIGZyZXNod2F0ZXIgd2l0aGRyYXdhbHMgYW5kIGlzIG9uZSBvZiB0aGUgbGFyZ2VzdCBjb25zdW1lcnMgb2YgZW5lcmd5IGluIG1hbnkgcmVnaW9ucyBbIDEsIDJdLiBUcmFkaXRpb25hbCBpcnJpZ2F0aW9uIHByYWN0aWNlcywgb2Z0ZW4gaW5lZmZpY2llbnQgYW5kIHJlc291cmNlLWludGVuc2l2ZSwgZXhhY2VyYmF0ZSB0aGVzZSBpc3N1ZXMsIGxlYWRpbmcgdG8gd2F0ZXIgd2FzdGFnZSwgZW5lcmd5IG92ZXJ1c2UsIGFuZCByZWR1Y2VkIGNyb3AgeWllbGRzLiJ9XQ%3D%3D.

This comprehensive review explores these innovations. It begins with historical context and limitations of traditional techniques, followed by detailed examinations of modern irrigation systems, including drip/micro-irrigation, sprinklers, and subsurface methods. Smart technologies like sensor-based scheduling, satellite imagery, IoT, and AI applications are analyzed next. Water management strategies—such as harvesting, wastewater recycling, desalination, deficit irrigation, and drought-resistant crops—are evaluated for their potential (Antu et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjMwLCJzdWJ0eXBlIjp7fSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiVXR0YW0gQmlzd2FzIiwiZmFtaWx5IjoiQW50dSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNZC4gU2FpZnVsIiwiZmFtaWx5IjoiSXNsYW0ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU3VqYXQiLCJmYW1pbHkiOiJBaG1lZCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNZCIsImZhbWlseSI6IkFyaWZ1enphbWFuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlMgQyBTaXJrYXIgYW5kIE4gTiIsImZhbWlseSI6IlNhaGEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUHVqYSBSYW5pIiwiZmFtaWx5IjoiTWl0dSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBbmltZXNoIiwiZmFtaWx5IjoiU2Fya2FyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5vciBBaWRhIiwiZmFtaWx5IjoiTWFoaWRkaW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiWnVsaGlsbWkiLCJmYW1pbHkiOiJJc21haWwifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiS2hhbGlkIEEuIiwiZmFtaWx5IjoiSWJyYWhpbSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBYnViYWtyIE0uIiwiZmFtaWx5IjoiSWRyaXMifX1dLCJpZHMiOnsiaXNzbiI6WyIyMjE0LTcxNDQiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMTYvai5qd3BlLjIwMjQuMTA2MzE3Iiwib3BlbmFsZXgiOiJXNDQwMzU2MjczOSIsImV4YSI6Imh0dHBzOi8vd3d3LnNjaWVuY2VkaXJlY3QuY29tL3NjaWVuY2UvYXJ0aWNsZS9hYnMvcGlpL1MyMjE0NzE0NDI0MDE1NDk2In0sInRpdGxlIjoiRW1lcmdpbmcgdGVjaG5vbG9naWVzIGZvciBlZmZpY2llbnQgd2F0ZXIgdXNlIGluIGFncmljdWx0dXJlOiBBIHJldmlldyBvZiBjdXJyZW50IHRyZW5kcyBhbmQgZnV0dXJlIGRpcmVjdGlvbnMiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAxNi9qLmp3cGUuMjAyNC4xMDYzMTciLCJpc3N1ZWREYXRlIjoiMjAyNC0xMC0xOSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTEiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkpvdXJuYWwgb2YgV2F0ZXIgUHJvY2VzcyBFbmdpbmVlcmluZyIsInZvbHVtZSI6NjgsImltcGFjdEZhY3RvciI6IjcuMTYifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjoxMDYzMTcsImxhc3RQYWdlIjoxMDYzMTd9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRWxzZXZpZXIgQlYifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjI1OSwiY2l0ZWRCeUNvdW50Ijo3fX0sInF1b3RlIjoiSG93ZXZlciwgY29udmVudGlvbmFsIHdhdGVyIG1hbmFnZW1lbnQgdGVjaG5pcXVlcyBhcmUgaW5zdWZmaWNpZW50IGZvciBlbWVyZ2luZyBjaGFsbGVuZ2VzLCBoZW5jZSB0aGUgbmVjZXNzaXR5IG9mIGFkb3B0aW5nIGFkdmFuY2VkIHRlY2hub2xvZ2llcy4gVGhlIGNvbmNlcHQgb2YgZW1lcmdlbnQgdGVjaG5vbG9naWVzIGluIHdhdGVyIG1hbmFnZW1lbnQgaW5jbHVkZXMgYSB2YXJpZXR5IG9mIG5vdmVsIGlkZWFzIHRoYXQgaW50ZW5kIHRvIG1ha2Ugd2F0ZXIgcmVzb3VyY2VzIG1vcmUgYXZhaWxhYmxlLCBzdXN0YWluYWJsZSwgYW5kIGVmZmljaWVudC4gQWR2YW5jZWQgc2Vuc29ycywgYXJ0aWZpY2lhbCBpbnRlbGxpZ2VuY2UsIHJlbW90ZSBzZW5zaW5nLCBuYW5vdGVjaG5vbG9neSwgYW5kIGRlc2FsaW5hdGlvbiBwcm9jZXNzaW5nIG1ldGhvZHMgYXJlIGFtb25nIHRoZXNlIHRlY2hub2xvZ2llcyBcXFsxMSwxMlxcXS4gVGhlIGFwcGxpY2F0aW9uIG9mIHN1Y2ggdGVjaG5vbG9naWVzIG1ha2VzIGZvciB0aGUgcmVhbC10aW1lIG1vbml0b3JpbmcgYW5kIGV4YWN0IGRpc3RyaWJ1dGlvbiBvZiByZXNvdXJjZXMgd2l0aCB0aGUgaGVscCBvZiBpbm5vdmF0aXZlIHRlY2huaXF1ZXMgYW5kIHdhdGVyIG1hbmFnZW1lbnQgc29sdXRpb25zLiIsInNjb3JlIjowLjE5NDM5OTk5NzU5MTk3MjM1LCJleHRlcm5hbENhY2hlSWQiOiJkZjRiMWQ2ZS1mMjA3LTMzZDItOGE0NC1jNTFkZGE3ZDY0MTgifV0%3D.

## Historical Context of Irrigation and Water Management

Irrigation has ancient origins, tracing back over 5,000 years to civilizations in Mesopotamia, Egypt, the Indus Valley, and China, where early societies engineered canals, ditches, and basins to divert river waters for flood-based agriculture. These innovations enabled surplus food production and societal growth but were rudimentary, relying on seasonal floods and manual labor(Wu et al., 2019)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNodWFuZ2xlaSIsImZhbWlseSI6Ild1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllvbmdwaW5nIiwiZmFtaWx5IjoiV2VpIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkJyaWFuIiwiZmFtaWx5IjoiSGVhZCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJZYW4iLCJmYW1pbHkiOiJaaGFvIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNjb3R0IiwiZmFtaWx5IjoiSGFubmEifX1dLCJpZHMiOnsiaXNzbiI6WyIyMDU1LTEwNDUiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwNTcvczQxNTk5LTAxOS0wMjgyLTEiLCJtYWciOiIyOTYxMjMyNjQwIiwib3BlbmFsZXgiOiJXMjk2MTIzMjY0MCIsIm9wZW5BY2Nlc3NJZCI6IjE3MzY1NjY5MTc4NDQ3Nzc4Mjk2In0sInRpdGxlIjoiVGhlIGRldmVsb3BtZW50IG9mIGFuY2llbnQgQ2hpbmVzZSBhZ3JpY3VsdHVyYWwgYW5kIHdhdGVyIHRlY2hub2xvZ3kgZnJvbSA4MDAwIEJDIHRvIDE5MTEgQUQiLCJhYnN0cmFjdCI6IkFic3RyYWN0IFRlY2hub2xvZ3kgZGV2ZWxvcG1lbnRzIGhhdmUgbWFkZSBzaWduaWZpY2FudCBpbXBhY3RzIG9uIGJvdGggaHVtYW5zIGFuZCB0aGUgZW52aXJvbm1lbnQgaW4gd2hpY2ggdGhleSBsaXZlLiBIb3dldmVyLCB0aGVyZSBpcyBsaW1pdGVkIHdob2xlLW9mLXN5c3RlbSB1bmRlcnN0YW5kaW5nIG9mIGFuY2llbnQgdGVjaG5vbG9neSBkZXZlbG9wbWVudC4gVGhpcyBwYXBlciBhaW1zIHRvIHVuY292ZXIgdGhlIGV2b2x1dGlvbmFyeSBwYXR0ZXJuIG9mIHRoZSBhbmNpZW50IENoaW5lc2UgYWdyaWN1bHR1cmFsIHRlY2hub2xvZ3kgc3lzdGVtIHRoYXQgZm9jdXNlZCBvbiBsYW5kIGFuZCB3YXRlciBtb2JpbGlzYXRpb25zIGZyb20gODAwMCBCQyB0byAxOTExIEFELiBPdXIgZmluZGluZ3Mgc2hvdyB0aGF0IGFncmljdWx0dXJhbCB0ZWNobm9sb2d5IGluIENoaW5hIHRyYW5zaXRpb25lZCB0aHJvdWdoIGFuIGV4dHJlbWVseSBzbG93LCBTLXNoYXBlZCBwYXRod2F5LCBpbmNyZWFzaW5nIG9ubHkgdGVuIGZvbGQgaW4gb3ZlciA4MDAwIHllYXJzLiBUaGUgdGVjaG5vbG9neSBzeXN0ZW0gd2FzIGluaXRpYWxseSBkcml2ZW4gYnkgdGFuZ2libGUgdG9vbHMgKDQwJSBvZiBncm93dGgpLCB0aGVuIGJ5IHRlY2hub2xvZ2ljYWwgdGhlb3JpZXMgYW5kIHByYWN0aWNlcyB0aGF0IGNvbnRyaWJ1dGVkIG1vcmUgdGhhbiA1MCUgb2YgZ3Jvd3RoLiBJdHMgZGV2ZWxvcG1lbnQgd2FzIHNwYXRpYWxseSBpbmNsaW5lZCB0byB0aGUgWWVsbG93IFJpdmVyIHRoZW4gdG8gdGhlIFlhbmd0emUgUml2ZXIgcmVnaW9uLCB3aGVyZSBvdmVyIDQ1JSBvZiB0ZWNobm9sb2dpZXMgd2VyZSBkZXZlbG9wZWQuIFRoaXMgc3R1ZHkgcHJvdmlkZXMgYW4gZW1waXJpY2FsIGJhc2VsaW5lIGZvciBjb21wYXJhdGl2ZSBzdHVkaWVzIGJldHdlZW4gcHJlLWluZHVzdHJpYWwgYW5kIGluZHVzdHJpYWwgdGVjaG5vbG9naWVzLiBHcmVhdGVyIHVuZGVyc3RhbmRpbmcgb2YgdGhlIG1lY2hhbmlzbXMgb2YgdGVjaG5vbG9neSBkZXZlbG9wbWVudCB3aWxsIGJlIHJlcXVpcmVkIHRvIHJlb3JpZW50YXRlIHRlY2hub2xvZ3kgZGV2ZWxvcG1lbnQgZm9yIHByZXNlbnQgYW5kIGZ1dHVyZSBnZW5lcmF0aW9ucy4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTA1Ny9zNDE1OTktMDE5LTAyODItMSIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cubmF0dXJlLmNvbS9hcnRpY2xlcy9zNDE1OTktMDE5LTAyODItMS5wZGYiLCJpc3N1ZWREYXRlIjoiMjAxOS0wNy0wOSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IlBhbGdyYXZlIENvbW11bmljYXRpb25zIiwiaXNzdWUiOjEsInZvbHVtZSI6NSwiaW1wYWN0RmFjdG9yIjoiMC4wMCJ9LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJQYWxncmF2ZSBNYWNtaWxsYW4ifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjQ0LCJjaXRlZEJ5Q291bnQiOjQ5fX0sInF1b3RlIjoiQWx0aG91Z2ggbW9zdCBvZiB0aGUgZW1wZXJvcnMgaW4gQ2hpbmEgY29uc2lkZXJlZCBhZ3JpY3VsdHVyZSB0byBiZSB0aGUgZm91bmRhLXRpb24gb2YgYSBzdGFibGUgc29jaWV0eSBhbmQgY2VudHJhbCB0byBzb2NpZXRhbCwgcG9saXRpY2FsIGFuZCBlY28tbm9taWMgYWN0aXZpdGllcywgbG9uZy10ZXJtIHN0YWJpbGl0eSBhbmQgZXh0cmVtZSBwb3dlciBjb25jZW50cmF0aW9uIGZhdm91cmVkIGV4cGVyaWVuY2UtYmFzZWQsIGxhYm91ci1pbnRlbnNpdmUgYWdyaS1jdWx0dXJhbCBwcmFjdGljZXMgb3ZlciBpbm5vdmF0aXZlIHRvb2xzIGFuZCBzY2llbnRpZmljIHRoZW9yaWVzLiBUaGlzIGV2ZW50dWFsbHkgcmVzdWx0ZWQgaW4gdGVjaG5vbG9naWNhbCBsb2NrLWluIGluIHRoZSBNUSBQZXJpb2QuIFRoaXMgc3R1ZHksIGZvciB0aGUgZmlyc3QgdGltZSwgZW1waXJpY2FsbHkgYW5kIHF1YW50aXRhLXRpdmVseSBvYnNlcnZlZCB0aGUgZGV2ZWxvcG1lbnQgb2YgYW4gYW5jaWVudCBhZ3JpY3VsdHVyYWwgdGVjaC1ub2xvZ3kgc3lzdGVtIGFuZCB1bmNvdmVyZWQgaXRzIHNwZWNpZmljIHRyYW5zaXRpb24gcGF0aHdheS4gQXMgaW4gYW5jaWVudCBDaGluYSwgb3RoZXIgYW5jaWVudCBjaXZpbGlzYXRpb25zIGdyYXBwbGVkIHdpdGggdGhlIGRldmVsb3BtZW50IG9mIGFncmljdWx0dXJlIChNYWNOZWlzaCwgMTk2NCkuIEFzIGFuY2llbnQgY2l2aWxpc2F0aW9ucyByZWxpZWQgb24gbGFyZ2Ugcml2ZXJzIHRvIGZsb3VyaXNoLCBuYXR1cmFsIGxhbmRzY2FwZXMgZGV0ZXJtaW5lZCB0aGUgdHlwZXMgb2YgYWdyaWN1bHR1cmFsIGFjdGl2aXRpZXMgYW5kIHRlY2hub2xvZ2llcyBhZG9wdGVkIChTYWRvcmkgZXQgYWwuLCAyMDEwKS4gUG90dGVyeSB0b29scyB3ZXJlIGNvbW1vbiBpbiB0aGUgTWVzb3BvdGFtaWFuIGNpdmlsaXNhdGlvbiBkdWUgdG8gcmljaCBjbGF5IHNvdXJjZXMgaW4gdGhlIFRpZ3Jpcy0gRXVwaHJhdGVzIFJpdmVyIEJhc2luLCB0aGUgYW5jaWVudCBFZ3lwdGlhbnMgZGV2ZWxvcGVkIGFkdmFuY2VkIGlycmlnYXRpb24gYW5kIGxhbmQgZGlzdHJpYnV0aW9uIHN5c3RlbXMgdG8gaGFybmVzcyB0aGUgTmlsZSBSaXZlciBmbG9vZGluZywgYW5kIGFuY2llbnQgSW5kaWFucyBmb2N1c2VkIG9uIGEgcGhpbG9zb3BoaWNhbCBodW1hbi1uYXR1cmUgcmVsYXRpb25zaGlwIGFsb25nIHRoZSBJbmR1cyBhbmQgR2FuZ2VzIHJpdmVycyAoTGl1LCAyMDE1KS4gRGlmZmVyZW50IHRlY2hub2xvZ2llcyBhZG9wdGVkIGluIHR1cm4gZ3JlYXRseSBpbmZsdWVuY2VkIHRoZSBkZWdyZWVzIG9mIHNvaWwgZXJvc2lvbiwgc2VkaW1lbnRhdGlvbiBhbmQgZmxvdyByZWdpbWUgY2hhbmdlcywgd2hpY2ggd2FzIGNvcnJlbGF0ZWQgY2xvc2VseSB0byB0aGUgbG9uZ2V2aXR5IG9mIGNpdmlsaXNhdGlvbnMgKER1bm5pbmcgZXQgYWwuLCAyMDEyOyBNb250Z29tZXJ5LCAyMDA3KS4gIiwic2NvcmUiOjAuMTYwODk5OTk2NzU3NTA3MzIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjczLjE0MjE1ODUwODMwMDc4LCJsZWZ0Ijo3LjMyMzQ3OTY1MjQwNDc4NSwid2lkdGgiOjQxLjY2NjkwODI2NDE2MDE1NiwiaGVpZ2h0IjoxLjMzNTU1OTk2NDE3OTk5Mjd9fSx7InBhZ2VOdW1iZXIiOjEzLCJiYm94Ijp7InRvcCI6NzQuNDgyNjgxMjc0NDE0MDYsImxlZnQiOjcuMzIzNDc4MjIxODkzMzEwNSwid2lkdGgiOjQxLjY2Njg5NjgyMDA2ODM2LCJoZWlnaHQiOjEuMzM1NTU5OTY0MTc5OTkyN319LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjo3NS44MTYwNjI5MjcyNDYxLCJsZWZ0Ijo3LjMyMzQ3ODIyMTg5MzMxMDUsIndpZHRoIjo0MS42NzA1NjI3NDQxNDA2MjUsImhlaWdodCI6MS4zMzU1NTk5NjQxNzk5OTI3fX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjc3LjE1NjU4NTY5MzM1OTM4LCJsZWZ0Ijo3LjMyMzQ3ODIyMTg5MzMxMDUsIndpZHRoIjo0LjAyMDk4NzUxMDY4MTE1MiwiaGVpZ2h0IjoxLjMzNTU1OTk2NDE3OTk5Mjd9fSx7InBhZ2VOdW1iZXIiOjEzLCJiYm94Ijp7InRvcCI6NzcuMTU2NTg1NjkzMzU5MzgsImxlZnQiOjEyLjg0NzI4ODEzMTcxMzg2Nywid2lkdGgiOjUuODcxNjg4MzY1OTM2Mjc5LCJoZWlnaHQiOjEuMzM1NTU5OTY0MTc5OTkyN319LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjo3Ny4xNTY1ODU2OTMzNTkzOCwibGVmdCI6MjAuMjI4MTU3MDQzNDU3MDMsIndpZHRoIjo2LjM1NTAzNjczNTUzNDY2OCwiaGVpZ2h0IjoxLjMzNTU1OTk2NDE3OTk5Mjd9fSx7InBhZ2VOdW1iZXIiOjEzLCJiYm94Ijp7InRvcCI6NzcuMTU2NTg1NjkzMzU5MzgsImxlZnQiOjI4LjA5NDc2MjgwMjEyNDAyMywid2lkdGgiOjQuOTgxMzE5OTA0MzI3MzkzLCJoZWlnaHQiOjEuMzM1NTU5OTY0MTc5OTkyN319LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjo3Ny4xNTY1ODU2OTMzNTkzOCwibGVmdCI6MzQuNTgwNDkzOTI3MDAxOTUsIndpZHRoIjoyLjM4MTc0NjA1MzY5NTY3ODcsImhlaWdodCI6MS4zMzU1NTk5NjQxNzk5OTI3fX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjc3LjE1NjU4NTY5MzM1OTM4LCJsZWZ0IjozOC40NjYxNzUwNzkzNDU3LCJ3aWR0aCI6NS4wNzgzMDU3MjEyODI5NTksImhlaWdodCI6MS4zMzU1NTk5NjQxNzk5OTI3fX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjc3LjE1NjU4NTY5MzM1OTM4LCJsZWZ0Ijo0NS4wNTY1MjIzNjkzODQ3NjYsIndpZHRoIjozLjkzODMxMDYyMzE2ODk0NTMsImhlaWdodCI6MS4zMzU1NTk5NjQxNzk5OTI3fX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjc4LjQ4OTk2NzM0NjE5MTQsImxlZnQiOjcuMzIzNDc4MjIxODkzMzEwNSwid2lkdGgiOjQxLjY3MjkzMTY3MTE0MjU4LCJoZWlnaHQiOjEuMzM1NTU5OTY0MTc5OTkyN319LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjo3OS44MzA0ODI0ODI5MTAxNiwibGVmdCI6Ny4zMjM0NzgyMjE4OTMzMTA1LCJ3aWR0aCI6NDEuNjY0NTI0MDc4MzY5MTQsImhlaWdodCI6MS4zMzU1NTk5NjQxNzk5OTI3fX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjgxLjE2Mzg2NDEzNTc0MjE5LCJsZWZ0Ijo3LjMyMzQ3ODIyMTg5MzMxMDUsIndpZHRoIjo0MS42NjczNzM2NTcyMjY1NiwiaGVpZ2h0IjoxLjMzNTU1OTk2NDE3OTk5Mjd9fSx7InBhZ2VOdW1iZXIiOjEzLCJiYm94Ijp7InRvcCI6ODIuNTA0NTA4OTcyMTY3OTcsImxlZnQiOjcuMzIzNDc4MjIxODkzMzEwNSwid2lkdGgiOjQxLjY3MTM0ODU3MTc3NzM0NCwiaGVpZ2h0IjoxLjMzNTU1OTk2NDE3OTk5Mjd9fSx7InBhZ2VOdW1iZXIiOjEzLCJiYm94Ijp7InRvcCI6ODMuODM3ODk4MjU0Mzk0NTMsImxlZnQiOjcuMzIzMzE3MDUwOTMzODM4LCJ3aWR0aCI6NDEuNjY2OTA4MjY0MTYwMTU2LCJoZWlnaHQiOjEuMzM1NTU5OTY0MTc5OTkyN319LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjo4NS4xNzg1MzU0NjE0MjU3OCwibGVmdCI6Ny4zMjMzMTcwNTA5MzM4MzgsIndpZHRoIjozOS4yOTE4MzE5NzAyMTQ4NDQsImhlaWdodCI6MS4zMzU1NTk5NjQxNzk5OTI3fX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjg2LjUxMTkxNzExNDI1NzgxLCJsZWZ0Ijo4LjkxMzkwNDE5MDA2MzQ3Nywid2lkdGgiOjQwLjA4MDQ0NDMzNTkzNzUsImhlaWdodCI6MS4zMzU1NTk5NjQxNzk5OTI3fX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjg3Ljg1MjQzOTg4MDM3MTEsImxlZnQiOjcuMzIzNDc1ODM3NzA3NTE5NSwid2lkdGgiOjQxLjY2OTYxMjg4NDUyMTQ4NCwiaGVpZ2h0IjoxLjMzNTU1OTk2NDE3OTk5Mjd9fSx7InBhZ2VOdW1iZXIiOjEzLCJiYm94Ijp7InRvcCI6ODkuMTg0MjcyNzY2MTEzMjgsImxlZnQiOjcuMzIzNDc4MjIxODkzMzEwNSwid2lkdGgiOjQxLjY2NzExODA3MjUwOTc2NiwiaGVpZ2h0IjoxLjMzNzExMjQyNjc1NzgxMjV9fSx7InBhZ2VOdW1iZXIiOjEzLCJiYm94Ijp7InRvcCI6OTAuNTI0Nzk1NTMyMjI2NTYsImxlZnQiOjcuMzIzODUwMTU0ODc2NzA5LCJ3aWR0aCI6NDEuNjY2OTAwNjM0NzY1NjI1LCJoZWlnaHQiOjEuMzM1NTU5OTY0MTc5OTkyN319LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjo5MS44NTgxNjk1NTU2NjQwNiwibGVmdCI6Ny4zMjM4NTAxNTQ4NzY3MDksIndpZHRoIjo0MS42NzEzNTYyMDExNzE4NzUsImhlaWdodCI6MS4zMzU1NTk5NjQxNzk5OTI3fX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjkzLjE5ODY5OTk1MTE3MTg4LCJsZWZ0Ijo3LjMyMzg1MTU4NTM4ODE4NCwid2lkdGgiOjQxLjY3MDA4MjA5MjI4NTE1NiwiaGVpZ2h0IjoxLjMzNTU1OTk2NDE3OTk5Mjd9fSx7InBhZ2VOdW1iZXIiOjEzLCJiYm94Ijp7InRvcCI6Ni4yODU2MzgzMzIzNjY5NDMsImxlZnQiOjUwLjk5OTk4MDkyNjUxMzY3LCJ3aWR0aCI6NDEuNjY4MzM4Nzc1NjM0NzY2LCJoZWlnaHQiOjEuMzM1NTYwNDQxMDE3MTUwOX19LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjo3LjYyNjE2MDE0NDgwNTkwOCwibGVmdCI6NTAuOTk5OTgwOTI2NTEzNjcsIndpZHRoIjo0MS42NzE2OTE4OTQ1MzEyNSwiaGVpZ2h0IjoxLjMzNTU2MDQ0MTAxNzE1MDl9fSx7InBhZ2VOdW1iZXIiOjEzLCJiYm94Ijp7InRvcCI6OC45NTk1NDQxODE4MjM3MywibGVmdCI6NTAuOTk5OTgwOTI2NTEzNjcsIndpZHRoIjo0MS42Njk3OTU5ODk5OTAyMzQsImhlaWdodCI6MS4zMzU1NTk5NjQxNzk5OTI3fX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjEwLjMwMDA2NTk5NDI2MjY5NSwibGVmdCI6NTAuOTk5OTgwOTI2NTEzNjcsIndpZHRoIjo0MS42NzI5NDMxMTUyMzQzNzUsImhlaWdodCI6MS4zMzU1NTk5NjQxNzk5OTI3fX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjExLjY0MDU4NzgwNjcwMTY2LCJsZWZ0Ijo1MC45OTk5ODA5MjY1MTM2Nywid2lkdGgiOjQxLjY3MTM0NDc1NzA4MDA4LCJoZWlnaHQiOjEuMzM1NTU5OTY0MTc5OTkyN319LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjoxMi45NzM5NzA0MTMyMDgwMDgsImxlZnQiOjUwLjk5OTk4MDkyNjUxMzY3LCJ3aWR0aCI6NDEuNjcwNzE5MTQ2NzI4NTE2LCJoZWlnaHQiOjEuMzM1NTU5OTY0MTc5OTkyN319LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjoxNC4zMTQ2MDk1Mjc1ODc4OSwibGVmdCI6NTAuOTk5OTgwOTI2NTEzNjcsIndpZHRoIjo0MS42NzU1MjU2NjUyODMyLCJoZWlnaHQiOjEuMzM1NTU5OTY0MTc5OTkyN319LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjoxNS42NDc5OTIxMzQwOTQyMzgsImxlZnQiOjUwLjk5OTk4MDkyNjUxMzY3LCJ3aWR0aCI6NDEuNjc2NDYwMjY2MTEzMjgsImhlaWdodCI6MS4zMzU1NTk5NjQxNzk5OTI3fX1dfSwiY2h1bmtJZCI6Ijk2ZTU1NGJkLTc0MDAtNDVjNi1hYjI4LWY3YWFhNjc0YmYxYSIsImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjE0In19XQ%3D%3D.

Traditional surface irrigation methods—flooding, furrow, border, and wild flooding—evolved as dominant practices worldwide, forming the backbone of agriculture for millennia. However, these gravity-fed systems are inherently inefficient, prone to excessive evaporation, runoff, deep percolation, nonuniform distribution, and soil erosion (Walter et al., 2017)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjoxLCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkEuIiwiZmFtaWx5IjoiV2FsdGVyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlIuIiwiZmFtaWx5IjoiRmluZ2VyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlIuIiwiZmFtaWx5IjoiSHViZXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTi4iLCJmYW1pbHkiOiJCdWNobWFubiJ9fV0sImlkcyI6eyJpc3NuIjpbXSwiaXNibiI6W10sImRvaSI6IjEwLjEwNzMvcG5hcy4xNzA3NDYyMTE0IiwicG1pZCI6IjI4NjExMTk0IiwidXBsb2FkSWQiOiJZUkFQaE9xd0VwY2ZUZ1dSRkhzWCJ9LCJ0aXRsZSI6IlNtYXJ0IGZhcm1pbmcgaXMga2V5IHRvIGRldmVsb3Bpbmcgc3VzdGFpbmFibGUgYWdyaWN1bHR1cmUiLCJhYnN0cmFjdCI6IkFncmljdWx0dXJlIGhhcyBzZWVuIG1hbnkgcmV2b2x1dGlvbnMsIHdoZXRoZXIgdGhlIGRvbWVzdGljYXRpb24gb2YgYW5pbWFscyBhbmQgcGxhbnRzIGEgZmV3IHRob3VzYW5kIHllYXJzIGFnbywgdGhlIHN5c3RlbWF0aWMgdXNlIG9mIGNyb3Agcm90YXRpb25zIGFuZCBvdGhlciBpbXByb3ZlbWVudHMgaW4gZmFybWluZyBwcmFjdGljZSBhIGZldyBodW5kcmVkIHllYXJzIGFnbywgb3IgdGhlIOKAnGdyZWVuIHJldm9sdXRpb27igJ0gd2l0aCBzeXN0ZW1hdGljIGJyZWVkaW5nIGFuZCB0aGUgd2lkZXNwcmVhZCB1c2Ugb2YgbWFuLW1hZGUgZmVydGlsaXplcnMgYW5kIHBlc3RpY2lkZXMgYSBmZXcgZGVjYWRlcyBhZ28uIFdlIHN1Z2dlc3QgdGhhdCBhZ3JpY3VsdHVyZSBpcyB1bmRlcmdvaW5nIGEgZm91cnRoIHJldm9sdXRpb24gdHJpZ2dlcmVkIGJ5IHRoZSBleHBvbmVudGlhbGx5IGluY3JlYXNpbmcgdXNlIG9mIGluZm9ybWF0aW9uIGFuZCBjb21tdW5pY2F0aW9uIHRlY2hub2xvZ3kgKElDVCkgaW4gYWdyaWN1bHR1cmUuXG5cblxuXG5OZXcgdGVjaG5vbG9naWVzLCBzdWNoIGFzIHVubWFubmVkIGFlcmlhbCB2ZWhpY2xlcyB3aXRoIHBvd2VyZnVsLCBsaWdodHdlaWdodCBjYW1lcmFzLCBhbGxvdyBmb3IgaW1wcm92ZWQgZmFybSBtYW5hZ2VtZW50IGFkdmljZS4gSW1hZ2UgY291cnRlc3kgb2YgU2h1dHRlcnN0b2NrL0tsZWlyLlxuXG5cblxuQXV0b25vbW91cywgcm9ib3RpYyB2ZWhpY2xlcyBoYXZlIGJlZW4gZGV2ZWxvcGVkIGZvciBmYXJtaW5nIHB1cnBvc2VzLCBzdWNoIGFzIG1lY2hhbmljYWwgd2VlZGluZywgYXBwbGljYXRpb24gb2YgZmVydGlsaXplciwgb3IgaGFydmVzdGluZyBvZiBmcnVpdHMuIFRoZSBkZXZlbG9wbWVudCBvZiB1bm1hbm5lZCBhZXJpYWwgdmVoaWNsZXMgd2l0aCBhdXRvbm9tb3VzIGZsaWdodCBjb250cm9sICgxKSwgdG9nZXRoZXIgd2l0aCB0aGUgZGV2ZWxvcG1lbnQgb2YgbGlnaHR3ZWlnaHQgYW5kIHBvd2VyZnVsIGh5cGVyc3BlY3RyYWwgc25hcHNob3QgY2FtZXJhcyB0aGF0IGNhbiBiZSB1c2VkIHRvIGNhbGN1bGF0ZSBiaW9tYXNzIGRldmVsb3BtZW50IGFuZCBmZXJ0aWxpemF0aW9uIHN0YXR1cyBvZiBjcm9wcyAoMiwgMyksIG9wZW5zIHRoZSBmaWVsZCBmb3Igc29waGlzdGljYXRlZCBmYXJtIG1hbmFnZW1lbnQgYWR2aWNlLiBNb3Jlb3ZlciwgZGVjaXNpb24tdHJlZSBtb2RlbHMgYXJlIGF2YWlsYWJsZSBub3cgdGhhdCBhbGxvdyBmYXJtZXJzIHRvIGRpZmZlcmVudGlhdGUgYmV0d2VlbiBwbGFudCBkaXNlYXNlcyBiYXNlZCBvbiBvcHRpY2FsIGluZm9ybWF0aW9uICg0KS4gVmlydHVhbCBmZW5jZSB0ZWNobm9sb2dpZXMgKDUpIGFsbG93IGNhdHRsZSBoZXJkIG1hbmFnZW1lbnQgYmFzZWQgb24gcmVtb3RlLXNlbnNpbmcgc2lnbmFscyBhbmQgc2Vuc29ycyBvciBhY3R1YXRvcnMgYXR0YWNoZWQgdG8gdGhlIGxpdmVzdG9jay5cblxuVGFrZW4gdG9nZXRoZXIsIHRoZXNlIHRlY2huaWNhbCBpbXByb3ZlbWVudHMgY29uc3RpdHV0ZSBhIHRlY2huaWNhbCByZXZvbHV0aW9uIHRoYXQgd2lsbCBnZW5lcmF0ZSBkaXNydXB0aXZlIGNoYW5nZXMgaW4gYWdyaWN1bHR1cmFsIHByYWN0aWNlcy4gVGhpcyB0cmVuZCBob2xkcyBmb3IgZmFybWluZyBub3Qgb25seSBpbiBkZXZlbG9wZWQgY291bnRyaWVzIGJ1dCBhbHNvIGluIGRldmVsb3BpbmcgY291bnRyaWVzLCB3aGVyZSBkZXBsb3ltZW50cyBpbiBJQ1QgKGUuZy4sIHVzZSBvZiBtb2JpbGUgcGhvbmVzLCBhY2Nlc3MgdG8gdGhlIEludGVybmV0KSBhcmUgYmVpbmcgYWRvcHRlZCBhdCBhIHJhcGlkIHBhY2UgYW5kIGNvdWxkIGJlY29tZSB0aGUgZ2FtZS1jaGFuZ2VycyBpbiB0aGUgZnV0dXJlIChlLmcuLCBpbiB0aGUgZm9ybSBvZiBzZWFzb25hbCBkcm91Z2h0IGZvcmVjYXN0cywgY2xpbWF0ZS1zbWFydCBhZ3JpY3VsdHVyZSkuXG5cblN1Y2ggcHJvZm91bmQgY2hhbmdlcyBpbiBwcmFjdGljZSBjb21lIG5vdCBvbmx5IHdpdGggb3Bwb3J0dW5pdGllcyBidXQgYWxzbyBiaWcgY2hhbGxlbmdlcy4gSXQgaXMgY3J1Y2lhbCB0byBwb2ludCB0aGVtIG91dCBhdCBhbiBlYXJseSBzdGFnZSBvZiB0aGlzIOKApiBcblxuW%2BKGtV1bMV0xVG8gd2hvbSBjb3JyZXNwb25kZW5jZSBzaG91bGQgYmUgYWRkcmVzc2VkLiBFbWFpbDogYWNoaW0ud2FsdGVye2F0fXVzeXMuZXRoei5jaC5cblxuIFsxXTogI3hyZWYtY29ycmVzcC0xLTEiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTA3My9wbmFzLjE3MDc0NjIxMTQiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vd3d3LnBuYXMub3JnL2NvbnRlbnQvcG5hcy8xMTQvMjQvNjE0OC5mdWxsLnBkZiIsImlzc3VlZERhdGUiOiIyMDE3IiwiY29sbGVjdGlvbkluZm8iOnsiYXV0aG9ycyI6W10sImVkaXRvcnMiOltdfSwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJQcm9jLiBOYXQuIEFjYWQuIFNjaS4gVVNBIiwidm9sdW1lIjoxMTR9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjYxNDgsImxhc3RQYWdlIjo2MTUwfSwibnVtYmVyIjoyNH0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6Ik5hdGlvbmFsIEFjYWRlbXkgb2YgU2NpZW5jZXMifSwiZXh0cmEiOnsiZmllbGRzIjp7fX19LCJzY29yZSI6MC4xNzE4MDAwMDI0NTU3MTEzNn1d. Globally, agriculture consumes approximately 70% of freshwater withdrawals through such methods, while also driving high energy demands and contributing to environmental degradation (Gomes et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjMwLCJzdWJ0eXBlIjp7fSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTWVnYW4iLCJmYW1pbHkiOiJHb21lcyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJUaW1vdGh5IEouIiwiZmFtaWx5IjoiUmFscGgifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTWFyYyIsImZhbWlseSI6Ikh1bXBocmllcyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJCcmFkbGV5IFAuIiwiZmFtaWx5IjoiR3JhdmVzIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRzdXlvc2hpIiwiZmFtaWx5IjoiS29iYXlhc2hpIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkRhbWlhbiBCLiIsImZhbWlseSI6IkdvcmUifX1dLCJpZHMiOnsiaXNzbiI6WyIwMDQ4LTk2OTciLCIxODc5LTEwMjYiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMTYvai5zY2l0b3RlbnYuMjAyNC4xNzgwODQiLCJwbWlkIjoiMzk2NzQxNDgiLCJvcGVuYWxleCI6Ilc0NDA1MzU4Nzk5IiwiZXhhIjoiaHR0cHM6Ly93d3cuc2NpZW5jZWRpcmVjdC5jb20vc2NpZW5jZS9hcnRpY2xlL3BpaS9TMDA0ODk2OTcyNDA4MjQyMSJ9LCJ0aXRsZSI6IldhdGVyYm9ybmUgY29udGFtaW5hbnRzIGluIGhpZ2ggaW50ZW5zaXR5IGFncmljdWx0dXJlIGFuZCBwbGFudCBwcm9kdWN0aW9uOiBBIHJldmlldyBvZiBvbi1zaXRlIGFuZCBkb3duc3RyZWFtIGltcGFjdHMiLCJhYnN0cmFjdCI6IldhdGVyYm9ybmUgY29udGFtaW5hbnRzIHBvc2UgYSBzaWduaWZpY2FudCByaXNrIHRvIHdhdGVyIHF1YWxpdHkgYW5kIHBsYW50IGhlYWx0aCBpbiBhZ3JpY3VsdHVyYWwgc3lzdGVtcy4gVGhpcyBpcyBwYXJ0aWN1bGFybHkgdGhlIGNhc2UgZm9yIHJlbGF0aXZlbHkgc21hbGwtc2NhbGUgYnV0IGludGVuc2l2ZSBhZ3JpY3VsdHVyYWwgb3BlcmF0aW9ucyBzdWNoIGFzIHBsYW50IHByb2R1Y3Rpb24gbnVyc2VyaWVzIHRoYXQgb2Z0ZW4gcmVseSBvbiByZWN5Y2xlZCBpcnJpZ2F0aW9uIHdhdGVyLiBUaGUgaW5jcmVhc2luZyBnbG9iYWwgZGVtYW5kIGZvciBwbGFudHMgcmVxdWlyZXMgaW1wcm92ZWQgd2F0ZXIgcXVhbGl0eSBhbmQgbW9yZSBjZXJ0YWludHkgYXJvdW5kIHdhdGVyIGF2YWlsYWJpbGl0eSwgd2hpY2ggbWF5IGJlIGRpZmZpY3VsdCB0byBwcmVkaWN0IGFuZCBkZWxpdmVyIGR1ZSB0byB2YXJpYWJsZSBhbmQgY2hhbmdpbmcgY2xpbWF0ZSByZWdpbWVzLiBQcm9kdWN0aW9uIG51cnNlcmllcyBhcmUgbW92aW5nIHRvIGFkb3B0IGJlc3QgbWFuYWdlbWVudCBwcmFjdGljZXMgdGhhdCByZWN5Y2xlIHdhdGVyOyBob3dldmVyLCB0aGUgcmlza3MgYXNzb2NpYXRlZCB3aXRoIHdhdGVyYm9ybmUgY29udGFtaW5hbnRzIG9mIHZhcmlvdXMgdHlwZXMsIGluY2x1ZGluZyBudXRyaWVudHMsIHBlc3RpY2lkZXMsIHBsYW50IHBhdGhvZ2VucywgbWljcm8tcGxhc3RpY3MsIGFuZCB0b3hpYyBtZXRhbHMsIGFyZSBub3Qgd2VsbCB1bmRlcnN0b29kLiBXZSByZXZpZXcgYW5kIHN5bnRoZXNpc2UgdGhlIHBoeXNpY2FsIGFuZCBiaW9nZW9jaGVtaWNhbCBmYWN0b3JzIHRoYXQgY29udHJpYnV0ZSB0byB3YXRlcmJvcm5lIGNvbnRhbWluYW50IHJpc2ssIGFuZCB0aGUgbWFpbiB0eXBlcyBvZiBjb250YW1pbmFudHMgdGhhdCBhcmUgbGlrZWx5IHRvIHJlcXVpcmUgbWFuYWdlbWVudCwgYXQgcGxhbnQgcHJvZHVjdGlvbiBudXJzZXJpZXMuIENhdGNobWVudCBjaGFyYWN0ZXJpc3RpY3MgKGkuZS4sIHRvcG9ncmFwaHksIGxhbmQgdXNlKSwgaHlkcm9jbGltYXRpYyBmYWN0b3JzIChpLmUuLCBzdG9ybXMsIGZsb29kcywgZHJvdWdodHMpLCBhbmQgbGFuZHNjYXBlIGh5ZHJvbG9naWNhbCBhbmQgc2VkaW1lbnQgY29ubmVjdGl2aXR5IGluZmx1ZW5jZSBzdXJmYWNlIHJ1bm9mZiwgc2VkaW1lbnQgdHJhbnNwb3J0LCBhbmQgYXNzb2NpYXRlZCBjb250YW1pbmFudCB0cmFuc2ZlciBhbmQgc3RvcmFnZS4gSGlnaCBoeWRyb2xvZ2ljYWwgY29ubmVjdGl2aXR5IGNhbiBpbmNyZWFzZSB0aGUgcmlzayBvZiBjb250YW1pbmFudCB0cmFuc3BvcnQgZnJvbSB0aGUgc3Vycm91bmRpbmcgbGFuZHNjYXBlIHRvIG51cnNlcmllcywgd2l0aCBwb3RlbnRpYWwgbmVnYXRpdmUgaW1wYWN0cyB0byB3YXRlciBxdWFsaXR5IGluIHJlc2Vydm9pcnMgYW5kIGluIHR1cm4gcGxhbnQgaGVhbHRoLiBIaWdoIGNvbm5lY3Rpdml0eSBtYXkgYWxzbyBpbmNyZWFzZSB0aGUgcmlzayBvZiBjb250YW1pbmFudHMgKGUuZy4sIHNlZGltZW50LCBwZXN0aWNpZGVzLCBhbmQgcGh5dG9wYXRob2dlbnMpIGJlaW5nIHRyYW5zZmVycmVkIGZyb20gbnVyc2VyeSBmYXJtcyBpbnRvIGRvd25zdHJlYW0gd2F0ZXJ3YXlzLCB3aXRoIGNvbnNlcXVlbmNlcyBmb3IgYXF1YXRpYyBlY29zeXN0ZW1zLiBMaWtlIGFsbCBpbnRlbnNpdmUgYWdyaWN1bHR1cmFsIG9wZXJhdGlvbnMsIG51cnNlcmllcyBuZWVkIHRvIGNvbnNpZGVyIHNvdXJjZXMgb2YgaXJyaWdhdGlvbiB3YXRlciwgd2F0ZXIgdHJlYXRtZW50IGFuZCBtYW5hZ2VtZW50IHN0cmF0ZWdpZXMsIGFuZCBjYXRjaG1lbnQgYW5kIGh5ZHJvY2xpbWF0aWMgZmFjdG9ycywgdG8gbWl0aWdhdGUgdGhlIHNwcmVhZCBvZiBjb250YW1pbmFudHMgYW5kIHJlZHVjZSB0aGVpciBpbXBhY3RzIG9uIGJvdGggcGxhbnQgcHJvZHVjdGlvbiBhbmQgdGhlIHN1cnJvdW5kaW5nIGVudmlyb25tZW50LiBGdXJ0aGVyIHJlc2VhcmNoIGlzIG5lZWRlZCB0byBxdWFudGlmeSBjb250YW1pbmFudCBsb2FkcyBhbmQgdHJhbnNmZXIgcGF0aHdheXMgaW4gdGhlc2UgYWdyaWN1bHR1cmFsIHN5c3RlbXMsIGFuZCB0byBiZXR0ZXIgdW5kZXJzdGFuZCB0aGUgdGhyZXNob2xkIGxldmVscyBvZiBjb250YW1pbmFudHMgdGhhdCBhZHZlcnNlbHkgYWZmZWN0IHBsYW50IGhlYWx0aCBhbmQgd2hpY2ggbWF5IHJlc3VsdCBpbiBkZXZhc3RhdGluZyBlY29ub21pYyBsb3NzZXMuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjEwMTYvai5zY2l0b3RlbnYuMjAyNC4xNzgwODQiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2ouc2NpdG90ZW52LjIwMjQuMTc4MDg0IiwiaXNzdWVkRGF0ZSI6IjIwMjQtMTItMTMiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTExIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJUaGUgU2NpZW5jZSBvZiBUaGUgVG90YWwgRW52aXJvbm1lbnQiLCJ2b2x1bWUiOjk1OCwiaW1wYWN0RmFjdG9yIjoiMTEuOTQifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjoxNzgwODQsImxhc3RQYWdlIjoxNzgwODR9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRWxzZXZpZXIgQlYifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjE0NCwiY2l0ZWRCeUNvdW50Ijo4fX0sInF1b3RlIjoiW0FncmljdWx0dXJlXShodHRwczovL3d3dy5zY2llbmNlZGlyZWN0LmNvbS90b3BpY3MvYWdyaWN1bHR1cmFsLWFuZC1iaW9sb2dpY2FsLXNjaWVuY2VzL2FncmljdWx0dXJhbC1zY2llbmNlKSBhY2NvdW50cyBmb3IgYXBwcm94aW1hdGVseSA3MMKgJSBvZiB0aGUgd29ybGQncyBmcmVzaHdhdGVyIHJlc291cmNlIHdpdGhkcmF3YWxzICggW0Z1Z2xpZSBldCBhbC4sIDIwMjRdKGh0dHBzOi8vd3d3LnNjaWVuY2VkaXJlY3QuY29tL3NjaWVuY2UvYXJ0aWNsZS9waWkvUzAwNDg5Njk3MjQwODI0MjEjYmIwMTkwKSkuIFdoaWxlIGxhcmdlIHF1YW50aXRpZXMgb2Ygd2F0ZXIgYXJlIHVzZWQgZm9yIFthZ3JpY3VsdHVyYWwgaXJyaWdhdGlvbl0oaHR0cHM6Ly93d3cuc2NpZW5jZWRpcmVjdC5jb20vdG9waWNzL2FncmljdWx0dXJhbC1hbmQtYmlvbG9naWNhbC1zY2llbmNlcy9hZ3JpY3VsdHVyYWwtaXJyaWdhdGlvbiksIHN1YnN0YW50aWFsIHByb3BvcnRpb25zIGFyZSBsb3N0IHRvIHRoZSBlbnZpcm9ubWVudCB0aHJvdWdoIHJ1bm9mZi4gR2l2ZW4gY29uc3RyYWludHMgb24gZ2xvYmFsIHdhdGVyIGF2YWlsYWJpbGl0eSwgZHVlIHRvIHBvcHVsYXRpb24gZ3Jvd3RoIGFuZCBjbGltYXRlIHZhcmlhYmlsaXR5LCBpbiBjb25qdW5jdGlvbiB3aXRoIGluY3JlYXNpbmcgcHVibGljIGFuZCByZWd1bGF0b3J5IHByZXNzdXJlLCBwcm9kdWNlcnMgaGF2ZSBiZWVuIGRyaXZlbiB0byBjYXB0dXJlIGFuZCByZWN5Y2xlIHJ1bm9mZiBmb3IgaXJyaWdhdGlvbiB0byBlbnN1cmUgdGhlIHN1c3RhaW5hYmlsaXR5IG9mIGFncmljdWx0dXJhbCBwcm9kdWN0aW9uICggW09icmV6YSBldCBhbC4sIDIwMTBdKGh0dHBzOi8vd3d3LnNjaWVuY2VkaXJlY3QuY29tL3NjaWVuY2UvYXJ0aWNsZS9waWkvUzAwNDg5Njk3MjQwODI0MjEjYmIwNDQ1KTsgW1poYW5nIGV0IGFsLiwgMjAxNV0oaHR0cHM6Ly93d3cuc2NpZW5jZWRpcmVjdC5jb20vc2NpZW5jZS9hcnRpY2xlL3BpaS9TMDA0ODk2OTcyNDA4MjQyMSNiYjA3MjApOyBbUG91ZHlhbCBhbmQgQ3JlZ2csIDIwMTldKGh0dHBzOi8vd3d3LnNjaWVuY2VkaXJlY3QuY29tL3NjaWVuY2UvYXJ0aWNsZS9waWkvUzAwNDg5Njk3MjQwODI0MjEjYmIwNTAwKSkuIiwic2NvcmUiOjAuMTg3ODAwMDA1MDc4MzE1NzMsImV4dGVybmFsQ2FjaGVJZCI6IjA3YzZkZjdiLTI4NjQtM2ZmMC1iY2NhLTZkYWZkNDMzNTg5NCJ9XQ%3D%3D. These entrenched limitations—water wastage, energy overuse, and suboptimal yields—have intensified under modern pressures like population growth and climate variability, compelling a paradigm shift toward precision and sustainable alternatives (Daraz et al., 2025)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlVtYXIiLCJmYW1pbHkiOiJEYXJheiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiLFoHRlZmFuIiwiZmFtaWx5IjoiQm9qbmVjIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllvdW5hcyIsImZhbWlseSI6IktoYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMDc3LTA0NzIiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzOTAvYWdyaWN1bHR1cmUxNTA1MDU1NCIsIm9wZW5hbGV4IjoiVzQ0MDgyMTk4MTEiLCJleGEiOiJodHRwczovL3d3dy5tZHBpLmNvbS8yMDc3LTA0NzIvMTUvNS81NTQifSwidGl0bGUiOiJFbmVyZ3ktRWZmaWNpZW50IFNtYXJ0IElycmlnYXRpb24gVGVjaG5vbG9naWVzOiBBIFBhdGh3YXkgdG8gV2F0ZXIgYW5kIEVuZXJneSBTdXN0YWluYWJpbGl0eSBpbiBBZ3JpY3VsdHVyZSIsImFic3RyYWN0IjoiVGhlIGFncmljdWx0dXJhbCBzZWN0b3IgZmFjZXMgY2hhbGxlbmdlcyBzdWNoIGFzIHdhdGVyIHNjYXJjaXR5LCBlbmVyZ3kgaW5lZmZpY2llbmN5LCBhbmQgZGVjbGluaW5nIHByb2R1Y3Rpdml0eSwgcGFydGljdWxhcmx5IGluIGFyaWQgcmVnaW9ucy4gVHJhZGl0aW9uYWwgaXJyaWdhdGlvbiBtZXRob2RzIGNvbnRyaWJ1dGUgdG8gcmVzb3VyY2UgZGVwbGV0aW9uIGFuZCBlbnZpcm9ubWVudGFsIGltcGFjdHMuIFNvbGFyLXBvd2VyZWQgc21hcnQgaXJyaWdhdGlvbiBzeXN0ZW1zIGludGVncmF0ZSBwcmVjaXNpb24gaXJyaWdhdGlvbiB3aXRoIHJlbmV3YWJsZSBlbmVyZ3ksIGltcHJvdmluZyB3YXRlciB1c2UgYW5kIHByb2R1Y3Rpdml0eS4gSW4gUGFraXN0YW4sIHdoZXJlIGFncmljdWx0dXJlIGNvbnRyaWJ1dGVzIDE5JSBvZiBncm9zcyBkb21lc3RpYyBwcm9kdWN0IGFuZCBlbXBsb3lzIDQwJSBvZiB0aGUgd29ya2ZvcmNlLCB0aGVzZSBjaGFsbGVuZ2VzIGFyZSBzZXZlcmUsIGVzcGVjaWFsbHkgaW4gd2F0ZXItc2NhcmNlIGFyZWFzIGxpa2UgdGhlIENob2xpc3RhbiBEZXNlcnQuIFRoaXMgc3R1ZHkgZXhhbWluZXMgdGhlIGltcGFjdCBvZiBzb2xhci1wb3dlcmVkIHNtYXJ0IGlycmlnYXRpb24gb24gYWdyaWN1bHR1cmFsIHByb2R1Y3Rpdml0eSwgd2F0ZXIgY29uc2VydmF0aW9uLCBhbmQgZW5lcmd5IGVmZmljaWVuY3kgaW4gdGhlIENob2xpc3RhbiBEZXNlcnQuIFVzaW5nIGEgcXVhbnRpdGF0aXZlIGNyb3NzLXNlY3Rpb25hbCBkZXNpZ24sIGRhdGEgd2VyZSBjb2xsZWN0ZWQgZnJvbSAzODQgZmFybWVycyB2aWEgc3RydWN0dXJlZCBxdWVzdGlvbm5haXJlcy4gU3RhdGlzdGljYWwgYW5hbHlzZXMsIGluY2x1ZGluZyBtdWx0aXBsZSBsaW5lYXIgcmVncmVzc2lvbiwgcGFpcmVkIHNhbXBsZSB0LXRlc3RzLCBhbmQgU3RydWN0dXJhbCBFcXVhdGlvbiBNb2RlbGluZyAoU0VNKSwgd2VyZSBjb25kdWN0ZWQuIEZpbmRpbmdzIHNob3cgc2lnbmlmaWNhbnQgaW1wcm92ZW1lbnRzIGluIGNyb3AgeWllbGQgKGZyb20gMy4wIHRvIDQuOCB0b25zL2hlY3RhcmUpIGFuZCByZWR1Y3Rpb25zIGluIHdhdGVyIGFuZCBlbmVyZ3kgY29uc3VtcHRpb24uIFJlZ3Jlc3Npb24gYW5hbHlzaXMgaGlnaGxpZ2h0ZWQgc3Ryb25nIHBvc2l0aXZlIGVmZmVjdHMgb24geWllbGQgYW5kIGVmZmljaWVuY3ksIHdoaWxlIFNFTSBjb25maXJtZWQgcmVkdWNlZCBlbnZpcm9ubWVudGFsIGltcGFjdCBhbmQgb3BlcmF0aW9uYWwgY29zdHMuIFRoZSBzdHVkeSBjb25jbHVkZXMgdGhhdCBzb2xhci1wb3dlcmVkIGlycmlnYXRpb24gZW5oYW5jZXMgcHJvZHVjdGl2aXR5LCBjb25zZXJ2ZXMgcmVzb3VyY2VzLCBhbmQgcHJvbW90ZXMgc3VzdGFpbmFiaWxpdHkuIFBvbGljeW1ha2VycyBzaG91bGQgcHJvdmlkZSBmaW5hbmNpYWwgaW5jZW50aXZlcywgaW52ZXN0IGluIHJlbmV3YWJsZSBpbmZyYXN0cnVjdHVyZSwgYW5kIGltcGxlbWVudCB0cmFpbmluZyBwcm9ncmFtcyB0byBzdXBwb3J0IGFkb3B0aW9uLiBDb2xsYWJvcmF0aXZlIGVmZm9ydHMgYXJlIGVzc2VudGlhbCBmb3Igc3VzdGFpbmFibGUgYWdyaWN1bHR1cmUgaW4gYXJpZCByZWdpb25zLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJpc3N1ZWREYXRlIjoiMjAyNS0wMy0wNSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkFncmljdWx0dXJlIiwiaXNzdWUiOjUsInZvbHVtZSI6MTUsImltcGFjdEZhY3RvciI6IjUuMjYifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjo1NTQsImxhc3RQYWdlIjo1NTR9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiTXVsdGlkaXNjaXBsaW5hcnkgRGlnaXRhbCBQdWJsaXNoaW5nIEluc3RpdHV0ZSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NDgsImNpdGVkQnlDb3VudCI6Mn19LCJxdW90ZSI6IjEuIEludHJvZHVjdGlvbiBUaGUgYWdyaWN1bHR1cmFsIHNlY3RvciwgYSBjb3JuZXJzdG9uZSBvZiBnbG9iYWwgZm9vZCBzZWN1cml0eSBhbmQgZWNvbm9taWMgZGV2ZWxvcG1lbnQsIGZhY2VzIG1vdW50aW5nIGNoYWxsZW5nZXMgaW4gdGhlIDIxc3QgY2VudHVyeS4gQ2xpbWF0ZSBjaGFuZ2UsIHJhcGlkIHBvcHVsYXRpb24gZ3Jvd3RoLCBhbmQgZHdpbmRsaW5nIG5hdHVyYWwgcmVzb3VyY2VzIGhhdmUgaW50ZW5zaWZpZWQgdGhlIG5lZWQgZm9yIHN1c3RhaW5hYmxlIGFncmljdWx0dXJhbCBwcmFjdGljZXMuIEFtb25nIHRoZXNlIGNoYWxsZW5nZXMsIHdhdGVyIHNjYXJjaXR5IGFuZCBlbmVyZ3kgaW5lZmZpY2llbmN5IHN0YW5kIG91dCBhcyBjcml0aWNhbCBiYXJyaWVycyB0byBwcm9kdWN0aXZpdHkgYW5kIHN1c3RhaW5hYmlsaXR5LiBHbG9iYWxseSwgYWdyaWN1bHR1cmUgYWNjb3VudHMgZm9yIGFwcHJveGltYXRlbHkgNzAlIG9mIGZyZXNod2F0ZXIgd2l0aGRyYXdhbHMgYW5kIGlzIG9uZSBvZiB0aGUgbGFyZ2VzdCBjb25zdW1lcnMgb2YgZW5lcmd5IGluIG1hbnkgcmVnaW9ucyBbIDEsIDJdLiBUcmFkaXRpb25hbCBpcnJpZ2F0aW9uIHByYWN0aWNlcywgb2Z0ZW4gaW5lZmZpY2llbnQgYW5kIHJlc291cmNlLWludGVuc2l2ZSwgZXhhY2VyYmF0ZSB0aGVzZSBpc3N1ZXMsIGxlYWRpbmcgdG8gd2F0ZXIgd2FzdGFnZSwgZW5lcmd5IG92ZXJ1c2UsIGFuZCByZWR1Y2VkIGNyb3AgeWllbGRzLiJ9XQ%3D%3D.

## Traditional Irrigation Techniques and Their Limitations

Surface irrigation techniques—predominantly flood, furrow, border, and basin methods—have historically dominated agricultural water application worldwide, relying on gravity to spread water across fields. While enabling large-scale cultivation, these approaches are fundamentally inefficient, resulting in substantial water wastage, energy overuse, and reduced crop productivity, as evidenced by agriculture's consumption of ~70% of global freshwater alongside high energy demands (Qin et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjozLCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikppbmd4aXUiLCJmYW1pbHkiOiJRaW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiV2VpbGkiLCJmYW1pbHkiOiJEdWFuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNoYW4iLCJmYW1pbHkiOiJab3UifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiWWFuaW5nIiwiZmFtaWx5IjoiQ2hlbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJXZW5qaW5nIiwiZmFtaWx5IjoiSHVhbmcifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTG9yZW56byIsImZhbWlseSI6IlJvc2EifX1dLCJpZHMiOnsiaXNzbiI6WyIyMDQxLTE3MjMiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMzgvczQxNDY3LTAyNC00NzM4My01IiwicG1pZCI6IjM4NjAwMDU5Iiwib3BlbmFsZXgiOiJXNDM5NDY4ODQ1NSIsImV4YSI6Imh0dHBzOi8vd3d3Lm5hdHVyZS5jb20vYXJ0aWNsZXMvczQxNDY3LTAyNC00NzM4My01P2Vycm9yPWNvb2tpZXNfbm90X3N1cHBvcnRlZCZjb2RlPTJlOTM5NjNiLTNjOTktNGQyNS05ZjI4LWE3YThiNmNjNzEyOSJ9LCJ0aXRsZSI6Ikdsb2JhbCBlbmVyZ3kgdXNlIGFuZCBjYXJib24gZW1pc3Npb25zIGZyb20gaXJyaWdhdGVkIGFncmljdWx0dXJlIiwiYWJzdHJhY3QiOiJBYnN0cmFjdCBJcnJpZ2F0aW9uIGlzIGEgbGFuZCBtYW5hZ2VtZW50IHByYWN0aWNlIHdpdGggbWFqb3IgZW52aXJvbm1lbnRhbCBpbXBhY3RzLiBIb3dldmVyLCBnbG9iYWwgZW5lcmd5IGNvbnN1bXB0aW9uIGFuZCBjYXJib24gZW1pc3Npb25zIHJlc3VsdGluZyBmcm9tIGlycmlnYXRpb24gcmVtYWluIHVua25vd24uIFdlIGFzc2VzcyB0aGUgd29ybGR3aWRlIGVuZXJneSBjb25zdW1wdGlvbiBhbmQgY2FyYm9uIGVtaXNzaW9ucyBhc3NvY2lhdGVkIHdpdGggaXJyaWdhdGlvbiwgd2hpbGUgYWxzbyBtZWFzdXJpbmcgdGhlIHBvdGVudGlhbCBlbmVyZ3kgYW5kIGNhcmJvbiByZWR1Y3Rpb25zIGFjaGlldmFibGUgdGhyb3VnaCB0aGUgYWRvcHRpb24gb2YgZWZmaWNpZW50IGFuZCBsb3ctY2FyYm9uIGlycmlnYXRpb24gcHJhY3RpY2VzLiBDdXJyZW50bHksIGlycmlnYXRpb24gY29udHJpYnV0ZXMgMjE2IG1pbGxpb24gbWV0cmljIHRvbnMgb2YgQ08gMiBlbWlzc2lvbnMgYW5kIGNvbnN1bWVzIDE4OTYgcGV0YWpvdWxlcyBvZiBlbmVyZ3kgYW5udWFsbHksIHJlcHJlc2VudGluZyAxNSUgb2YgZ3JlZW5ob3VzZSBnYXMgZW1pc3Npb25zIGFuZCBlbmVyZ3kgdXRpbGl6ZWQgaW4gYWdyaWN1bHR1cmFsIG9wZXJhdGlvbnMuIERlc3BpdGUgb25seSA0MCUgb2YgaXJyaWdhdGVkIGFncmljdWx0dXJlIHJlbGllcyBvbiBncm91bmR3YXRlciBzb3VyY2VzLCBncm91bmR3YXRlciBwdW1waW5nIGFjY291bnRzIGZvciA4OSUgb2YgdGhlIHRvdGFsIGVuZXJneSBjb25zdW1wdGlvbiBpbiBpcnJpZ2F0aW9uLiBQcm9qZWN0aW9ucyBpbmRpY2F0ZSB0aGF0IGZ1dHVyZSBleHBhbnNpb24gb2YgaXJyaWdhdGlvbiBjb3VsZCBsZWFkIHRvIGEgMjglIGluY3JlYXNlIGluIGVuZXJneSB1c2FnZS4gRW1icmFjaW5nIGhpZ2hseSBlZmZpY2llbnQsIGxvdy1jYXJib24gaXJyaWdhdGlvbiBtZXRob2RzIGhhcyB0aGUgcG90ZW50aWFsIHRvIGN1dCBlbmVyZ3kgY29uc3VtcHRpb24gaW4gaGFsZiBhbmQgcmVkdWNlIENPIDIgZW1pc3Npb25zIGJ5IDkwJS4gSG93ZXZlciwgY29uc2lkZXJpbmcgY291bnRyeS1zcGVjaWZpYyBmZWFzaWJpbGl0eSBvZiBtaXRpZ2F0aW9uIG9wdGlvbnMsIGdsb2JhbCBDTyAyIGVtaXNzaW9ucyBtYXkgb25seSBzZWUgYSA1NSUgcmVkdWN0aW9uLiBPdXIgcmVzZWFyY2ggb2ZmZXJzIGNvbXByZWhlbnNpdmUgaW5zaWdodHMgaW50byB0aGUgZW5lcmd5IGNvbnN1bXB0aW9uIGFuZCBjYXJib24gZW1pc3Npb25zIGFzc29jaWF0ZWQgd2l0aCBpcnJpZ2F0aW9uLCBjb250cmlidXRpbmcgdmFsdWFibGUgaW5mb3JtYXRpb24gdGhhdCBjYW4gZ3VpZGUgYXNzZXNzbWVudHMgb2YgdGhlIHZpYWJpbGl0eSBvZiBpcnJpZ2F0aW9uIGluIGVuaGFuY2luZyBhZGFwdGl2ZSBjYXBhY2l0eSB3aXRoaW4gdGhlIGFncmljdWx0dXJhbCBzZWN0b3IuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjEwMzgvczQxNDY3LTAyNC00NzM4My01IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5uYXR1cmUuY29tL2FydGljbGVzL3M0MTQ2Ny0wMjQtNDczODMtNS5wZGYiLCJpc3N1ZWREYXRlIjoiMjAyNC0wNC0xMCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjYtMDEiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6Ik5hdHVyZSBDb21tdW5pY2F0aW9ucyIsImlzc3VlIjoxLCJ2b2x1bWUiOjE1LCJpbXBhY3RGYWN0b3IiOiIxOS43MCJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjMwODQsImxhc3RQYWdlIjozMDg0fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6Ik5hdHVyZSBQb3J0Zm9saW8ifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjU0LCJjaXRlZEJ5Q291bnQiOjE0Mn19LCJxdW90ZSI6IlNldmVudHkgcGVyY2VudCBvZiB3b3JsZHdpZGUgd2F0ZXIgd2l0aGRyYXdhbHMgYW5kIDgw4oCTOTAlIG9mIHdhdGVyIGNvbnN1bXB0aW9uIGFyZSBhdHRyaWJ1dGVkIHRvIGlycmlnYXRlZCBhZ3JpY3VsdHVyZSAxIC4gSXJyaWdhdGlvbiBwbGF5cyBhIHBpdm90YWwgcm9sZSBpbiBlbnN1cmluZyBnbG9iYWwgZm9vZCBzZWN1cml0eSwgY29udHJpYnV0aW5nIHRvIDQwJSBvZiBnbG9iYWwgZm9vZCBwcm9kdWN0aW9uIHdoaWxlIHV0aWxpemluZyBvbmx5IDIyJSBvZiB0aGUgcGxhbmV04oCZcyBjdWx0aXZhdGVkIGFyZWFzIDIgLCAzIC4gQXMgdGhlIGNoYWxsZW5nZXMgb2YgZ2xvYmFsIHdhcm1pbmcgYW5kIHBvcHVsYXRpb24gZ3Jvd3RoIGludGVuc2lmeSwgZXhhY2VyYmF0aW5nIGNvbmNlcm5zIGFib3V0IHdhdGVyIGFuZCBmb29kIHNlY3VyaXR5LCBpcnJpZ2F0aW9uIGVtZXJnZXMgYXMgYSBjcnVjaWFsIGFkYXB0aXZlIG1lYXN1cmUgdG8gYWRkcmVzcyBmdXR1cmUgZm9vZCBjcmlzZXMgYW5kIHRoZSBpbXBhY3RzIG9mIGNsaW1hdGUgY2hhbmdlIDQgLiIsInNjb3JlIjowLjE5OTIwMDAwNDMzOTIxODE0fV0%3D.

Critical limitations include:

- Excessive water wastage through evaporation, runoff, deep percolation, and nonuniform distribution, with agriculture accounting for approximately 70% of global freshwater withdrawals (Mahmood et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjMwLCJzdWJ0eXBlIjp7fSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSGFpZGVyIiwiZmFtaWx5IjoiTWFobW9vZCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNdWhhbW1hZCBTaGFoaWQiLCJmYW1pbHkiOiJIYXNzYW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiR293aGFyIiwiZmFtaWx5IjoiTWVyYWoifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTWFoYW0iLCJmYW1pbHkiOiJGdXJxYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMjk3LTY0NzciXSwiaXNibiI6W10sImRvaSI6IjEwLjU2NTc4L2NpczEyMDMwMiIsIm9wZW5hbGV4IjoiVzQ0MDM4Mzg1NTciLCJvcGVuQWNjZXNzSWQiOiI1NTE3MjkwNDE2MjM2MzEwOTE2In0sInRpdGxlIjoiQWdyaWN1bHR1cmUncyBSb2xlIGluIEVudmlyb25tZW50YWwgU3VzdGFpbmFiaWxpdHk6IEEgQ29tcHJlaGVuc2l2ZSBSZXZpZXcgb2YgQ2hhbGxlbmdlcyBhbmQgU29sdXRpb25zIiwiYWJzdHJhY3QiOiJUaGUgZ3Jvd2luZyBnbG9iYWwgcG9wdWxhdGlvbiBoYXMgcGxhY2VkIGluY3JlYXNpbmcgcHJlc3N1cmUgb24gdGhlIGFncmljdWx0dXJlIHNlY3RvciB0byBtZWV0IHJpc2luZyBmb29kIGRlbWFuZCwgcG9zaW5nIHNpZ25pZmljYW50IGVudmlyb25tZW50YWwgYW5kIGVjb2xvZ2ljYWwgY2hhbGxlbmdlcy4gVGhpcyByZXZpZXcgc3lzdGVtYXRpY2FsbHkgZXhhbWluZXMgNzAgc3R1ZGllcyBzZWxlY3RlZCBmcm9tIHRoZSBTY29wdXMgZGF0YWJhc2UsIHdpdGggYSBmb2N1cyBvbiB0aGUgZW52aXJvbm1lbnRhbCBpbXBhY3RzIG9mIGFncmljdWx0dXJlIGFuZCBwb3RlbnRpYWwgbWl0aWdhdGlvbiBzdHJhdGVnaWVzLiBPZiB0aGUgNzAgYXJ0aWNsZXMsIDM4IHN0dWRpZXMgZXhwbG9yZSB0aGUgbWFjcm9lY29ub21pYyBlbnZpcm9ubWVudGFsIGVmZmVjdHMgb2YgYWdyaWN1bHR1cmUuIFdoaWxlIDEwIHN0dWRpZXMgcmVwb3J0IHBvc2l0aXZlIGVudmlyb25tZW50YWwgY29udHJpYnV0aW9ucyBmcm9tIHRoZSBzZWN0b3IsIDIzIGhpZ2hsaWdodCBhZHZlcnNlIGVjb2xvZ2ljYWwgY29uc2VxdWVuY2VzLiBBZGRpdGlvbmFsbHksIHZhcmlvdXMgc3R1ZGllcyBpbmRpY2F0ZSBVLXNoYXBlZCwgaW52ZXJ0ZWQgVS1zaGFwZWQsIG9yIE4tc2hhcGVkIHJlbGF0aW9uc2hpcHMgYmV0d2VlbiBhZ3JpY3VsdHVyYWwgYWN0aXZpdGllcyBhbmQgcG9sbHV0aW9uIGxldmVscy4gTGl2ZXN0b2NrIHByb2R1Y3Rpb24gYW5kIHRoZSBleHRlbnNpdmUgdXNlIG9mIHN5bnRoZXRpYyBmZXJ0aWxpc2VycyBhcmUgaWRlbnRpZmllZCBhcyBtYWpvciBjb250cmlidXRvcnMgdG8gZ3JlZW5ob3VzZSBnYXMgKEdIRykgZW1pc3Npb25zLCB3aGlsZSB0aGUgd2lkZXNwcmVhZCB1c2Ugb2YgcGVzdGljaWRlcyBhbmQgaGVyYmljaWRlcyBoYXMgYmVlbiBzaG93biB0byBjYXVzZSBzb2lsIGFuZCB3YXRlciBjb250YW1pbmF0aW9uLiBGdXJ0aGVyIGVudmlyb25tZW50YWwgZGVncmFkYXRpb24gaXMgbGlua2VkIHRvIGRlZm9yZXN0YXRpb24gZHJpdmVuIGJ5IGFncmljdWx0dXJhbCBleHBhbnNpb24sIHdoaWNoIHJlZHVjZXMgY2FyYm9uIHNpbmtzIGFuZCBiaW9kaXZlcnNpdHkuIFRoZSBhZ3JpY3VsdHVyZSBzZWN0b3IncyBkZXBlbmRlbmNlIG9uIGZvc3NpbCBmdWVscyBhbHNvIGV4YWNlcmJhdGVzIGl0cyBHSEcgZW1pc3Npb25zLCB3aGlsZSBpdHMgc2lnbmlmaWNhbnQgZnJlc2h3YXRlciBjb25zdW1wdGlvbiBoZWlnaHRlbnMgY29uY2VybnMgYWJvdXQgd2F0ZXIgc2NhcmNpdHkuIE1vcmVvdmVyLCBzb2lsIGRlZ3JhZGF0aW9uLCBvZnRlbiByZXN1bHRpbmcgZnJvbSBtb25vY3JvcHBpbmcgYW5kIGNvbnZlbnRpb25hbCBmYXJtaW5nIHByYWN0aWNlcywgcHJlc2VudHMgYW4gb25nb2luZyBjaGFsbGVuZ2UuIEhvd2V2ZXIsIHN1c3RhaW5hYmxlIGFncmljdWx0dXJhbCBwcmFjdGljZXMsIHN1Y2ggYXMgYWdyb2ZvcmVzdHJ5LCBjcm9wIHJvdGF0aW9uLCBjb25zZXJ2YXRpb24gdGlsbGFnZSwgYW5kIG9yZ2FuaWMgZmFybWluZywgb2ZmZXIgcHJvbWlzaW5nIHNvbHV0aW9ucyB0byBtaXRpZ2F0ZSB0aGVzZSBlbnZpcm9ubWVudGFsIGltcGFjdHMuIFRoZXNlIHByYWN0aWNlcyBub3Qgb25seSBlbmhhbmNlIHNvaWwgaGVhbHRoIGJ5IHJlZHVjaW5nIGNoZW1pY2FsIGlucHV0cyBidXQgYWxzbyBwcm9tb3RlIGJpb2RpdmVyc2l0eSB3aXRoaW4gZmFybWluZyBzeXN0ZW1zLiBQcmVjaXNpb24gYWdyaWN1bHR1cmUsIG9wdGltaXNhdGlvbiBvZiB3YXRlciwgZmVydGlsaXNlciwgYW5kIHBlc3RpY2lkZSB1c2FnZSwgdGhlIGFkb3B0aW9uIG9mIG5hdGl2ZSBwbGFudCBzcGVjaWVzLCBhbmQgdGhlIGludGVncmF0aW9uIG9mIHJlbmV3YWJsZSBlbmVyZ3kgc291cmNlcyBoYXZlIGJlZW4gaWRlbnRpZmllZCBhcyBrZXkgc3RyYXRlZ2llcyBmb3IgaW1wcm92aW5nIHRoZSBzdXN0YWluYWJpbGl0eSBvZiBhZ3JpY3VsdHVyYWwgb3BlcmF0aW9ucy4gQWRkaXRpb25hbGx5LCBnZW5ldGljIGFkdmFuY2VtZW50cyBpbiBjcm9wIGRldmVsb3BtZW50IG1heSBwbGF5IGEgY3JpdGljYWwgcm9sZSBpbiBhZGRyZXNzaW5nIHRoZSBzZWN0b3IncyBlbnZpcm9ubWVudGFsIGZvb3RwcmludC4gQnkgYWRvcHRpbmcgdGhlc2Ugc3VzdGFpbmFibGUgbWV0aG9kcywgdGhlIGFncmljdWx0dXJlIHNlY3RvciBoYXMgdGhlIHBvdGVudGlhbCB0byBpbmNyZWFzZSBwcm9kdWN0aXZpdHkgd2hpbGUgc2lnbmlmaWNhbnRseSByZWR1Y2luZyBpdHMgZW52aXJvbm1lbnRhbCBpbXBhY3QsIGNvbnRyaWJ1dGluZyB0byB0aGUgb3ZlcmFsbCBnb2FsIG9mIGVjb2xvZ2ljYWwgc3VzdGFpbmFiaWxpdHkuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjU2NTc4L2NpczEyMDMwMiIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9saWJyYXJ5LmFjYWRsb3JlLmNvbS9DSVMvMjAyNC8xMi8zL0NJU18xMi4wM18wMi5wZGYiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMC0yOCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDgiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkNoYWxsZW5nZXMgaW4gU3VzdGFpbmFiaWxpdHkiLCJpc3N1ZSI6Mywidm9sdW1lIjoxMiwiaW1wYWN0RmFjdG9yIjoiMS41OCJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjE3OCwibGFzdFBhZ2UiOjE4OX19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJMaWJyZWxsb3BoIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjowLCJjaXRlZEJ5Q291bnQiOjV9fSwicXVvdGUiOiJJbiBhZGRpdGlvbiwgZGVmb3Jlc3RhdGlvbiB0byBpbmNyZWFzZSBhZ3JpY3VsdHVyYWwgbGFuZCB3b3VsZCByZWR1Y2UgY2FyYm9uIHNpbmtzLCAgY291bGQgZGFtYWdlIGJpb2RpdmVyc2l0eSwgYW5kIHVwc2V0IHRoZSBiYWxhbmNlIG9mIHRoZSBlY29zeXN0ZW0uIFRoZSBzdHJlc3Mgb24gZ2xvYmFsIGZyZXNod2F0ZXIgcmVzb3VyY2VzIGlzICBhbm90aGVyIGNvbnNlcXVlbmNlIG9mIHRoZSBhZ3JpY3VsdHVyZSBzZWN0b3IsIGFzIHRoaXMgc2VjdG9yIGlzIHRoZSBsYXJnZXN0IHVzZXIgb2YgZnJlc2h3YXRlciBhbmQgaXMgcmVzcG9uc2libGUgZm9yICBmcmVzaHdhdGVyIHdpdGhkcmF3YWxzLiBUbyBtb2RlcmF0ZSB0aGUgYWR2ZXJzZSBlbnZpcm9ubWVudGFsIGVmZmVjdHMsIHRoZSBsaXRlcmF0dXJlIGhhcyBzdWdnZXN0ZWQgY292ZXIgIGNyb3BwaW5nLCBjb25zZXJ2YXRpb24gdGlsbGFnZSwgYWdyb2ZvcmVzdHJ5LCBpbnRlZ3JhdGluZyBuYXRpdmUgcGxhbnRzLCByZXN0b3JpbmcgZGVncmFkZWQgbGFuZHMsIGNyb3Agcm90YXRpb24sIGFuZCAgb3JnYW5pYyBmYXJtaW5nIHRvIGVuaGFuY2Ugc29pbCBwcm9kdWN0aXZpdHksIHJlZHVjdGlvbiBpbiBjaGVtaWNhbCB1c2FnZSwgYW5kIHByb21vdGlvbiBvZiBiaW9kaXZlcnNpdHkuICBNb3Jlb3ZlciwgaW5jcmVhc2luZyBhZ3JpY3VsdHVyYWwgcHJvZHVjdGl2aXR5LCBieSBvcHRpbWl6aW5nIHRoZSB1c2Ugb2Ygd2F0ZXIsIGZlcnRpbGl6ZXJzLCBhbmQgcGVzdGljaWRlcywgY2FuIGhlbHAgIHRoaXMgc2VjdG9yIHJlZHVjZSBlY29sb2dpY2FsIGFuZCBlbnZpcm9ubWVudGFsIHByb2JsZW1zLiBUaGUgaW1wcm92ZWQgbGl2ZXN0b2NrIGZlZWQgcXVhbGl0eSBhbmQgYmlvZW5lcmd5ICBnZW5lcmF0aW9uIGZyb20gY3JvcCByZXNpZHVlcyBhbmQgYW5pbWFsIHdhc3RlIHdvdWxkIGhlbHAgaW4gcmVkdWNpbmcgR0hHIGVtaXNzaW9ucy4gUmVwbGFjaW5nIHJlbmV3YWJsZSAgZW5lcmd5IHdpdGggZm9zc2lsIGZ1ZWxzIGFuZCBpbmNyZWFzaW5nIG92ZXJhbGwgZW5lcmd5IGVmZmljaWVuY3kgY2FuIGFsc28gaGVscCByZWR1Y2UgZW52aXJvbm1lbnRhbCBwcm9ibGVtcyBpbiAgdGhlIGFncmljdWx0dXJlIHNlY3Rvci4gSW4gYWRkaXRpb24sIHNvbWUgbW9kZXJuIHRlY2hub2xvZ2llcyBjb3VsZCBoZWxwIGltcHJvdmUgdGhlIGVudmlyb25tZW50YWwgcGVyZm9ybWFuY2UgIG9mIHRoZSBhZ3JpY3VsdHVyZSBzZWN0b3IuIEZvciBpbnN0YW5jZSwgcHJlY2lzaW9uIGFncmljdWx0dXJlIHdpdGggdGhlIGhlbHAgb2YgZHJvbmVzIGFuZCBzZW5zb3JzIGNvdWxkIHJlZHVjZSB1c2FnZSAgb2YgZmFybSBpbnB1dHMgYW5kIGltcHJvdmUgY3JvcCB5aWVsZC4gU2ltaWxhcmx5LCBnZW5ldGljIGFkdmFuY2VtZW50cyBjYW4gaGVscCBjcm9wcyByZXNpc3QgcGVzdHMsIGRpc2Vhc2VzLCBhbmQgIGNsaW1hdGljIHN0cmVzc2VzLCB3aGljaCBjYW4gcmVkdWNlIHRoZSB1c2FnZSBvZiBjaGVtaWNhbCBpbnB1dHMgYW5kIGVudmlyb25tZW50YWwgcHJvYmxlbXMuICIsInNjb3JlIjowLjE5NDQ5OTk5OTI4NDc0NDI2LCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjMwLjE0NTYyOTg4MjgxMjUsImxlZnQiOjExLjkxNjk1MjEzMzE3ODcxMSwid2lkdGgiOjc2LjUwODYyMTIxNTgyMDMxLCJoZWlnaHQiOjEuNTc4MTM3MDQwMTM4MjQ0Nn19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjMxLjUxMzkzNTA4OTExMTMyOCwibGVmdCI6MTEuOTE2OTUyMTMzMTc4NzExLCJ3aWR0aCI6NzYuNTAyNjI0NTExNzE4NzUsImhlaWdodCI6MS41NzgxMzcwNDAxMzgyNDQ2fX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6MzIuODgyMjMyNjY2MDE1NjI1LCJsZWZ0IjoxMS45MTY5NTIxMzMxNzg3MTEsIndpZHRoIjo3Ni41NDQ0MjU5NjQzNTU0NywiaGVpZ2h0IjoxLjU3ODEzNzA0MDEzODI0NDZ9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjozNC4yMzYyODIzNDg2MzI4MSwibGVmdCI6MTEuOTE2OTUyMTMzMTc4NzExLCJ3aWR0aCI6NzYuNTA1MjU2NjUyODMyMDMsImhlaWdodCI6MS41NzgxMzcwNDAxMzgyNDQ2fX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6MzUuNjA0NTgzNzQwMjM0Mzc1LCJsZWZ0IjoxMS45MTY5NTIxMzMxNzg3MTEsIndpZHRoIjo3Ni41NjMyNzgxOTgyNDIxOSwiaGVpZ2h0IjoxLjU3ODEzNzA0MDEzODI0NDZ9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjozNi45NzI4ODEzMTcxMzg2NywibGVmdCI6MTEuOTE2OTUyMTMzMTc4NzExLCJ3aWR0aCI6NzYuNTYzMTk0Mjc0OTAyMzQsImhlaWdodCI6MS41NzgxMzcwNDAxMzgyNDQ2fX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6MzguMzQxMTg2NTIzNDM3NSwibGVmdCI6MTEuOTE2OTUyMTMzMTc4NzExLCJ3aWR0aCI6NzYuNTIzNjQzNDkzNjUyMzQsImhlaWdodCI6MS41NzgxMzcwNDAxMzgyNDQ2fX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6MzkuNzA5NDg3OTE1MDM5MDYsImxlZnQiOjExLjkxNjk1MjEzMzE3ODcxMSwid2lkdGgiOjc2LjYwMjAyMDI2MzY3MTg4LCJoZWlnaHQiOjEuNTc4MTM3MDQwMTM4MjQ0Nn19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjQxLjA2MzUzMzc4Mjk1ODk4NCwibGVmdCI6MTEuOTE2OTUyMTMzMTc4NzExLCJ3aWR0aCI6NzYuNDk1MjM5MjU3ODEyNSwiaGVpZ2h0IjoxLjU3ODEzNzA0MDEzODI0NDZ9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo0Mi40MzE4MzUxNzQ1NjA1NSwibGVmdCI6MTEuOTE2OTUyMTMzMTc4NzExLCJ3aWR0aCI6NzYuNTA4NTkwNjk4MjQyMTksImhlaWdodCI6MS41NzgxMzcwNDAxMzgyNDQ2fX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NDMuODAwMTMyNzUxNDY0ODQ0LCJsZWZ0IjoxMS45MTY5NTIxMzMxNzg3MTEsIndpZHRoIjo3Ni41NzkzNjg1OTEzMDg2LCJoZWlnaHQiOjEuNTc4MTM3MDQwMTM4MjQ0Nn19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjQ1LjE2ODQzNzk1Nzc2MzY3LCJsZWZ0IjoxMS45MTY5NTIxMzMxNzg3MTEsIndpZHRoIjo3Ni41NzI4MDczMTIwMTE3MiwiaGVpZ2h0IjoxLjU3ODEzNzA0MDEzODI0NDZ9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo0Ni41MzY3MzkzNDkzNjUyMzQsImxlZnQiOjExLjkxNjk1MjEzMzE3ODcxMSwid2lkdGgiOjc2LjU5MTU2Nzk5MzE2NDA2LCJoZWlnaHQiOjEuNTc4MTM3MDQwMTM4MjQ0Nn19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjQ3LjkwNzQxMzQ4MjY2NjAxNiwibGVmdCI6MTEuOTE2OTUyMTMzMTc4NzExLCJ3aWR0aCI6NzYuNjAwMzAzNjQ5OTAyMzQsImhlaWdodCI6MS41NzgxMzcwNDAxMzgyNDQ2fX1dfSwiY2h1bmtJZCI6IjAzYmMwMTVmLTNiNzctNDM4Mi1hOGU3LTVjNzA4ZDczNGFjNyIsImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjE4NSJ9fV0%3D;

- High energy overuse, positioning agriculture as one of the largest energy consumers in many regions (Daraz et al., 2025)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlVtYXIiLCJmYW1pbHkiOiJEYXJheiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiLFoHRlZmFuIiwiZmFtaWx5IjoiQm9qbmVjIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllvdW5hcyIsImZhbWlseSI6IktoYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMDc3LTA0NzIiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzOTAvYWdyaWN1bHR1cmUxNTA1MDU1NCIsIm9wZW5hbGV4IjoiVzQ0MDgyMTk4MTEiLCJleGEiOiJodHRwczovL3d3dy5tZHBpLmNvbS8yMDc3LTA0NzIvMTUvNS81NTQifSwidGl0bGUiOiJFbmVyZ3ktRWZmaWNpZW50IFNtYXJ0IElycmlnYXRpb24gVGVjaG5vbG9naWVzOiBBIFBhdGh3YXkgdG8gV2F0ZXIgYW5kIEVuZXJneSBTdXN0YWluYWJpbGl0eSBpbiBBZ3JpY3VsdHVyZSIsImFic3RyYWN0IjoiVGhlIGFncmljdWx0dXJhbCBzZWN0b3IgZmFjZXMgY2hhbGxlbmdlcyBzdWNoIGFzIHdhdGVyIHNjYXJjaXR5LCBlbmVyZ3kgaW5lZmZpY2llbmN5LCBhbmQgZGVjbGluaW5nIHByb2R1Y3Rpdml0eSwgcGFydGljdWxhcmx5IGluIGFyaWQgcmVnaW9ucy4gVHJhZGl0aW9uYWwgaXJyaWdhdGlvbiBtZXRob2RzIGNvbnRyaWJ1dGUgdG8gcmVzb3VyY2UgZGVwbGV0aW9uIGFuZCBlbnZpcm9ubWVudGFsIGltcGFjdHMuIFNvbGFyLXBvd2VyZWQgc21hcnQgaXJyaWdhdGlvbiBzeXN0ZW1zIGludGVncmF0ZSBwcmVjaXNpb24gaXJyaWdhdGlvbiB3aXRoIHJlbmV3YWJsZSBlbmVyZ3ksIGltcHJvdmluZyB3YXRlciB1c2UgYW5kIHByb2R1Y3Rpdml0eS4gSW4gUGFraXN0YW4sIHdoZXJlIGFncmljdWx0dXJlIGNvbnRyaWJ1dGVzIDE5JSBvZiBncm9zcyBkb21lc3RpYyBwcm9kdWN0IGFuZCBlbXBsb3lzIDQwJSBvZiB0aGUgd29ya2ZvcmNlLCB0aGVzZSBjaGFsbGVuZ2VzIGFyZSBzZXZlcmUsIGVzcGVjaWFsbHkgaW4gd2F0ZXItc2NhcmNlIGFyZWFzIGxpa2UgdGhlIENob2xpc3RhbiBEZXNlcnQuIFRoaXMgc3R1ZHkgZXhhbWluZXMgdGhlIGltcGFjdCBvZiBzb2xhci1wb3dlcmVkIHNtYXJ0IGlycmlnYXRpb24gb24gYWdyaWN1bHR1cmFsIHByb2R1Y3Rpdml0eSwgd2F0ZXIgY29uc2VydmF0aW9uLCBhbmQgZW5lcmd5IGVmZmljaWVuY3kgaW4gdGhlIENob2xpc3RhbiBEZXNlcnQuIFVzaW5nIGEgcXVhbnRpdGF0aXZlIGNyb3NzLXNlY3Rpb25hbCBkZXNpZ24sIGRhdGEgd2VyZSBjb2xsZWN0ZWQgZnJvbSAzODQgZmFybWVycyB2aWEgc3RydWN0dXJlZCBxdWVzdGlvbm5haXJlcy4gU3RhdGlzdGljYWwgYW5hbHlzZXMsIGluY2x1ZGluZyBtdWx0aXBsZSBsaW5lYXIgcmVncmVzc2lvbiwgcGFpcmVkIHNhbXBsZSB0LXRlc3RzLCBhbmQgU3RydWN0dXJhbCBFcXVhdGlvbiBNb2RlbGluZyAoU0VNKSwgd2VyZSBjb25kdWN0ZWQuIEZpbmRpbmdzIHNob3cgc2lnbmlmaWNhbnQgaW1wcm92ZW1lbnRzIGluIGNyb3AgeWllbGQgKGZyb20gMy4wIHRvIDQuOCB0b25zL2hlY3RhcmUpIGFuZCByZWR1Y3Rpb25zIGluIHdhdGVyIGFuZCBlbmVyZ3kgY29uc3VtcHRpb24uIFJlZ3Jlc3Npb24gYW5hbHlzaXMgaGlnaGxpZ2h0ZWQgc3Ryb25nIHBvc2l0aXZlIGVmZmVjdHMgb24geWllbGQgYW5kIGVmZmljaWVuY3ksIHdoaWxlIFNFTSBjb25maXJtZWQgcmVkdWNlZCBlbnZpcm9ubWVudGFsIGltcGFjdCBhbmQgb3BlcmF0aW9uYWwgY29zdHMuIFRoZSBzdHVkeSBjb25jbHVkZXMgdGhhdCBzb2xhci1wb3dlcmVkIGlycmlnYXRpb24gZW5oYW5jZXMgcHJvZHVjdGl2aXR5LCBjb25zZXJ2ZXMgcmVzb3VyY2VzLCBhbmQgcHJvbW90ZXMgc3VzdGFpbmFiaWxpdHkuIFBvbGljeW1ha2VycyBzaG91bGQgcHJvdmlkZSBmaW5hbmNpYWwgaW5jZW50aXZlcywgaW52ZXN0IGluIHJlbmV3YWJsZSBpbmZyYXN0cnVjdHVyZSwgYW5kIGltcGxlbWVudCB0cmFpbmluZyBwcm9ncmFtcyB0byBzdXBwb3J0IGFkb3B0aW9uLiBDb2xsYWJvcmF0aXZlIGVmZm9ydHMgYXJlIGVzc2VudGlhbCBmb3Igc3VzdGFpbmFibGUgYWdyaWN1bHR1cmUgaW4gYXJpZCByZWdpb25zLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJpc3N1ZWREYXRlIjoiMjAyNS0wMy0wNSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkFncmljdWx0dXJlIiwiaXNzdWUiOjUsInZvbHVtZSI6MTUsImltcGFjdEZhY3RvciI6IjUuMjYifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjo1NTQsImxhc3RQYWdlIjo1NTR9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiTXVsdGlkaXNjaXBsaW5hcnkgRGlnaXRhbCBQdWJsaXNoaW5nIEluc3RpdHV0ZSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NDgsImNpdGVkQnlDb3VudCI6Mn19LCJxdW90ZSI6IjEuIEludHJvZHVjdGlvbiBUaGUgYWdyaWN1bHR1cmFsIHNlY3RvciwgYSBjb3JuZXJzdG9uZSBvZiBnbG9iYWwgZm9vZCBzZWN1cml0eSBhbmQgZWNvbm9taWMgZGV2ZWxvcG1lbnQsIGZhY2VzIG1vdW50aW5nIGNoYWxsZW5nZXMgaW4gdGhlIDIxc3QgY2VudHVyeS4gQ2xpbWF0ZSBjaGFuZ2UsIHJhcGlkIHBvcHVsYXRpb24gZ3Jvd3RoLCBhbmQgZHdpbmRsaW5nIG5hdHVyYWwgcmVzb3VyY2VzIGhhdmUgaW50ZW5zaWZpZWQgdGhlIG5lZWQgZm9yIHN1c3RhaW5hYmxlIGFncmljdWx0dXJhbCBwcmFjdGljZXMuIEFtb25nIHRoZXNlIGNoYWxsZW5nZXMsIHdhdGVyIHNjYXJjaXR5IGFuZCBlbmVyZ3kgaW5lZmZpY2llbmN5IHN0YW5kIG91dCBhcyBjcml0aWNhbCBiYXJyaWVycyB0byBwcm9kdWN0aXZpdHkgYW5kIHN1c3RhaW5hYmlsaXR5LiBHbG9iYWxseSwgYWdyaWN1bHR1cmUgYWNjb3VudHMgZm9yIGFwcHJveGltYXRlbHkgNzAlIG9mIGZyZXNod2F0ZXIgd2l0aGRyYXdhbHMgYW5kIGlzIG9uZSBvZiB0aGUgbGFyZ2VzdCBjb25zdW1lcnMgb2YgZW5lcmd5IGluIG1hbnkgcmVnaW9ucyBbIDEsIDJdLiBUcmFkaXRpb25hbCBpcnJpZ2F0aW9uIHByYWN0aWNlcywgb2Z0ZW4gaW5lZmZpY2llbnQgYW5kIHJlc291cmNlLWludGVuc2l2ZSwgZXhhY2VyYmF0ZSB0aGVzZSBpc3N1ZXMsIGxlYWRpbmcgdG8gd2F0ZXIgd2FzdGFnZSwgZW5lcmd5IG92ZXJ1c2UsIGFuZCByZWR1Y2VkIGNyb3AgeWllbGRzLiJ9XQ%3D%3D;

- Reduced crop yields and productivity due to inefficient, resource-intensive practices (Saha et al., 2025)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkdvdXJhYiIsImZhbWlseSI6IlNhaGEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiRmFyaWhhIiwiZmFtaWx5IjoiU2hhaHJpbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJGYXJoYW4gSGFzaW4iLCJmYW1pbHkiOiJLaGFuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1vaGFtbWFkIEFsaSIsImZhbWlseSI6Ik1lc2hrYXQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQUtNIEFiZHVsIE1hbGVrIiwiZmFtaWx5IjoiQXphZCJ9fV0sImlkcyI6eyJpc3NuIjpbIjE5MzItNjIwMyJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMTM3MS9qb3VybmFsLnBvbmUuMDMxOTI2OCIsInBtaWQiOiI0MDEwMDkwNiIsIm9wZW5hbGV4IjoiVzQ0MDg1NjQwNzkiLCJleGEiOiJodHRwczovL2pvdXJuYWxzLnBsb3Mub3JnL3Bsb3NvbmUvYXJ0aWNsZT9pZD0xMC4xMzcxJTJGam91cm5hbC5wb25lLjAzMTkyNjgifSwidGl0bGUiOiJTbWFydCBJb1QtZHJpdmVuIHByZWNpc2lvbiBhZ3JpY3VsdHVyZTogTGFuZCBtYXBwaW5nLCBjcm9wIHByZWRpY3Rpb24sIGFuZCBpcnJpZ2F0aW9uIHN5c3RlbSIsImFic3RyYWN0IjoiQXMgdGhlIHdvcmxkIHBvcHVsYXRpb24gaXMgaW5jcmVhc2luZyBkYXkgYnkgZGF5LCBzbyBpcyB0aGUgbmVlZCBmb3IgbW9yZSBhZHZhbmNlZCBhdXRvbWF0ZWQgcHJlY2lzaW9uIGFncmljdWx0dXJlIHRvIG1lZXQgdGhlIGluY3JlYXNpbmcgZGVtYW5kcyBmb3IgZm9vZCB3aGlsZSBkZWNyZWFzaW5nIGxhYm9yIHdvcmsgYW5kIHNhdmluZyB3YXRlciBmb3IgY3JvcHMuIFJlY2VudGx5LCB0aGVyZSBoYXZlIGJlZW4gbWFueSBzdHVkaWVzIGRvbmUgaW4gdGhpcyBmaWVsZCwgYnV0IHZlcnkgZmV3IGRpc2N1c3MgaW1wbGVtZW50aW5nIHNtYXJ0IHRlY2hub2xvZ2llcyB0byBwcmVzZW50IGEgY29tYmluZWQgc3VzdGFpbmFibGUgZmFybWluZyBzeXN0ZW0uIEluIHRoaXMgYXJ0aWNsZSwgd2UgcHJlc2VudCBhIGNvbXBsZXRlIGludGVncmF0ZWQgZGVzaWduIG9mIGEgc21hcnQgSW9ULWJhc2VkIHN1aXRhYmxlIGFncmljdWx0dXJhbCBsYW5kIGFuZCBjcm9wIHNlbGVjdGlvbiwgYWxvbmcgd2l0aCBhbiBpcnJpZ2F0aW9uIHN5c3RlbSB1c2luZyBhZ3JpY3VsdHVyYWwgbWFwcGluZywgbWFjaGluZSBsZWFybmluZywgYW5kIGZ1enp5IGxvZ2ljIGZvciBwcmVjaXNpb24gYWdyaWN1bHR1cmUuIE11bHRpLXNwZWN0cmFsIGJhbmQgaW1hZ2VzIGZyb20gTGFuZHNhdC04IHNhdGVsbGl0ZSBpbWFnZXMgb2YgYSBjaG9zZW4gbGFuZCBhcmUgZW1wbG95ZWQgZnJvbSBVU0dTIEVhcnRoIFJlc291cmNlcyBPYnNlcnZhdGlvbiBhbmQgU2NpZW5jZSAoRVJPUykgQ2VudGVyIGZvciBleHRyYWN0aW5nIGluZGljZXMgdGhhdCBhcmUgdXNlZCBmb3IgYWdyaWN1bHR1cmFsIGFuYWx5c2lzLCBkZXRlcm1pbmluZyB0aGUgdmVnZXRhdGlvbiBpbmRleCwgd2F0ZXIgaW5kZXgsIGFuZCBzYWxpbml0eSBpbmRleCBvZiB0aGF0IGxhbmQgdXNpbmcgSy1tZWFucy4gRnVydGhlcm1vcmUsIGNyb3AgeWllbGQgaXMgcHJlZGljdGVkIHVzaW5nIExpbmVhciBSZWdyZXNzaW9uIGFuZCBSYW5kb20gRm9yZXN0LCBhY2hpZXZpbmcgYWNjdXJhY2llcyBvZiA5My40OSUgYW5kIDk1Ljg3JSwgcmVzcGVjdGl2ZWx5LCB3aGlsZSB1c2luZyBSTVNFIChSb290IE1lYW4gU3F1YXJlZCBFcnJvcikgYXMgdGhlIGxvc3MgZnVuY3Rpb24uIFRoZSBMU1RNIG1vZGVsIGlzIHVzZWQgZm9yIGhlYWx0aHkgdmVnZXRhdGlvbiBhcmVhIGZvcmVjYXN0aW5nIGhpZ2hsaWdodGluZyB0aGUgY2hhbmdlcyBvZiB0aGUgdmVnZXRhdGlvbiBhcmVhIG92ZXIgdGltZS4gU3VjaCBhbmFseXNpcyBoZWxwcyB0byBkZWNpZGUgd2hldGhlciB0aGF0IGxhbmQgaXMgc3VpdGFibGUgZm9yIGZhcm1pbmcgb3Igbm90LiBNdWx0aXBsZSBzb2lsLXBhcmFtZXRlciBtZWFzdXJpbmcgc2Vuc29ycyBhcmUgdXNlZCB0byBpZGVudGlmeSBzdWl0YWJsZSBjcm9wIGFuZCBmZXJ0aWxpemVyIHJlcXVpcmVtZW50cyBmb3IgdGhhdCBsYW5kIHVzaW5nIElvVCBhbmQgbWFjaGluZSBsZWFybmluZy4gVGhlIE1MIG1vZGVsLWJhc2VkIGNyb3AgcHJlZGljdGlvbiBzaG93ZWQgOTcuMzUlIGFjY3VyYWN5IHV0aWxpemluZyByYW5kb20gZm9yZXN0IGFsZ29yaXRobS4gRmluYWxseSwgYSBmdXp6eSBsb2dpYy1iYXNlZCBzb2xhci1wb3dlcmVkIGlycmlnYXRpb24gc3lzdGVtIGlzIHVzZWQgdG8gbW9uaXRvciB0aGUgd2F0ZXIgcmVxdWlyZW1lbnRzIG9mIHRob3NlIGNyb3BzIGFuZCBpcnJpZ2F0ZSB0aGVtIGFjY29yZGluZyB0byB0aGVpciBuZWVkcy4gVGhlIGV4cGVyaW1lbnRhbCByZXN1bHRzIGRlbW9uc3RyYXRlZCB0aGF0IGZ1enp5IGxvZ2ljIGhhcyBmYXN0ZXIgY2FsaWJyYXRpb24gcmF0ZSBvZiA2Ni4yMyUgYW5kIGhlbHBzIHRvIHNhdmUgYXJvdW5kIDYxJSB3YXRlciBpbiBjb21wYXJpc29uIHRvIGF2ZXJhZ2UgbG9naWMgYWxnb3JpdGhtLiBUaGUgaW1wbGVtZW50YXRpb24gb2YgYSBmdXp6eSBsb2dpYyBhbGdvcml0aG0gc2lnbmlmaWNhbnRseSBvcHRpbWl6ZWQgd2F0ZXIgdXNhZ2UgY29tcGFyZWQgdG8gdHJhZGl0aW9uYWwgbWFudWFsIGlycmlnYXRpb24gbWV0aG9kcy4gVGhlc2UgZmluZGluZ3MgaGlnaGxpZ2h0IHRoZSBlZmZlY3RpdmVuZXNzIG9mIGFkdmFuY2VkIGNvbXB1dGF0aW9uYWwgdGVjaG5pcXVlcyBpbiBlbmhhbmNpbmcgYWdyaWN1bHR1cmFsIHByYWN0aWNlcyBhbmQgcmVzb3VyY2UgbWFuYWdlbWVudC4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTM3MS9qb3VybmFsLnBvbmUuMDMxOTI2OCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9qb3VybmFscy5wbG9zLm9yZy9wbG9zb25lL2FydGljbGUvZmlsZT9pZD0xMC4xMzcxL2pvdXJuYWwucG9uZS4wMzE5MjY4JnR5cGU9cHJpbnRhYmxlIiwiaXNzdWVkRGF0ZSI6IjIwMjUtMDMtMTgiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTExIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJQTG9TIE9ORSIsImlzc3VlIjozLCJ2b2x1bWUiOjIwLCJpbXBhY3RGYWN0b3IiOiIzLjM2In0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlB1YmxpYyBMaWJyYXJ5IG9mIFNjaWVuY2UifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjQ1LCJjaXRlZEJ5Q291bnQiOjZ9fSwicXVvdGUiOiJGYXJtZXJzLCBsYWNraW5nIHByaW9yIGluZm9ybWF0aW9uLCBhcmUgZXhjZXNzaXZlbHkgYXBwbHlpbmcgZmVydGlsaXplcnMsIGluc2VjdGljaWRlcywgYW5kIGlycmlnYXRpb24gaW4gb3JkZXIgdG8gYWNoaWV2ZSBoaWdoZXIgY3JvcCB5aWVsZHMgXFxbIFs0XShodHRwczovL2pvdXJuYWxzLnBsb3Mub3JnL2pvdXJuYWxzLnBsb3Mub3JnI3BvbmUuMDMxOTI2OC5yZWYwMDQpXFxdLiBUaGlzIGV4Y2Vzc2l2ZSB1c2FnZSBkZXBsZXRlcyByZXNvdXJjZXMsIGRlZ3JhZGVzIHNvaWwgcXVhbGl0eSwgYW5kIGFmZmVjdHMgYmlvZGl2ZXJzaXR5LiBBcHByb3hpbWF0ZWx5IDg3LjIxJSBvZiB0aGUgZW50aXJlIGZyZXNod2F0ZXIgc3VwcGx5IGlzIGFsbG9jYXRlZCBmb3IgdXNlIGluIHRoZSBhZ3JpY3VsdHVyYWwgaW5kdXN0cnkgXFxbIFs1XShodHRwczovL2pvdXJuYWxzLnBsb3Mub3JnL2pvdXJuYWxzLnBsb3Mub3JnI3BvbmUuMDMxOTI2OC5yZWYwMDUpXFxdLiBUaGlzIGhpZ2hsaWdodHMgdGhlIHVucGxhbm5lZCB1dGlsaXphdGlvbiBvZiB3YXRlciBpbiBpcnJpZ2F0aW9uLiBFbmVyZ3kgaXMgYWxzbyBhbiBpbXBvcnRhbnQgZmFjdG9yIGluIGlycmlnYXRpb24uIENvbnZlbnRpb25hbCB3YXRlciBwdW1wcyB1c2VkIGZvciBpcnJpZ2F0aW9uIGFyZSB1c3VhbGx5IHBvd2VyZWQgYnkgZWl0aGVyIGEgZGllc2VsIGdlbmVyYXRvciBvciBncmlkIGVuZXJneS4iLCJzY29yZSI6MC4xNTU2OTk5OTgxNDAzMzUwOCwiZXh0ZXJuYWxDYWNoZUlkIjoiMzU0NmE3YTctMmNkMC0zMTg5LWFkZjAtY2NhMDYzYTQ1Y2QyIn1d;

- Soil degradation, including soil erosion and contributions to broader sustainability challenges (Mahmood et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjMwLCJzdWJ0eXBlIjp7fSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSGFpZGVyIiwiZmFtaWx5IjoiTWFobW9vZCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNdWhhbW1hZCBTaGFoaWQiLCJmYW1pbHkiOiJIYXNzYW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiR293aGFyIiwiZmFtaWx5IjoiTWVyYWoifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTWFoYW0iLCJmYW1pbHkiOiJGdXJxYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMjk3LTY0NzciXSwiaXNibiI6W10sImRvaSI6IjEwLjU2NTc4L2NpczEyMDMwMiIsIm9wZW5hbGV4IjoiVzQ0MDM4Mzg1NTciLCJvcGVuQWNjZXNzSWQiOiI1NTE3MjkwNDE2MjM2MzEwOTE2In0sInRpdGxlIjoiQWdyaWN1bHR1cmUncyBSb2xlIGluIEVudmlyb25tZW50YWwgU3VzdGFpbmFiaWxpdHk6IEEgQ29tcHJlaGVuc2l2ZSBSZXZpZXcgb2YgQ2hhbGxlbmdlcyBhbmQgU29sdXRpb25zIiwiYWJzdHJhY3QiOiJUaGUgZ3Jvd2luZyBnbG9iYWwgcG9wdWxhdGlvbiBoYXMgcGxhY2VkIGluY3JlYXNpbmcgcHJlc3N1cmUgb24gdGhlIGFncmljdWx0dXJlIHNlY3RvciB0byBtZWV0IHJpc2luZyBmb29kIGRlbWFuZCwgcG9zaW5nIHNpZ25pZmljYW50IGVudmlyb25tZW50YWwgYW5kIGVjb2xvZ2ljYWwgY2hhbGxlbmdlcy4gVGhpcyByZXZpZXcgc3lzdGVtYXRpY2FsbHkgZXhhbWluZXMgNzAgc3R1ZGllcyBzZWxlY3RlZCBmcm9tIHRoZSBTY29wdXMgZGF0YWJhc2UsIHdpdGggYSBmb2N1cyBvbiB0aGUgZW52aXJvbm1lbnRhbCBpbXBhY3RzIG9mIGFncmljdWx0dXJlIGFuZCBwb3RlbnRpYWwgbWl0aWdhdGlvbiBzdHJhdGVnaWVzLiBPZiB0aGUgNzAgYXJ0aWNsZXMsIDM4IHN0dWRpZXMgZXhwbG9yZSB0aGUgbWFjcm9lY29ub21pYyBlbnZpcm9ubWVudGFsIGVmZmVjdHMgb2YgYWdyaWN1bHR1cmUuIFdoaWxlIDEwIHN0dWRpZXMgcmVwb3J0IHBvc2l0aXZlIGVudmlyb25tZW50YWwgY29udHJpYnV0aW9ucyBmcm9tIHRoZSBzZWN0b3IsIDIzIGhpZ2hsaWdodCBhZHZlcnNlIGVjb2xvZ2ljYWwgY29uc2VxdWVuY2VzLiBBZGRpdGlvbmFsbHksIHZhcmlvdXMgc3R1ZGllcyBpbmRpY2F0ZSBVLXNoYXBlZCwgaW52ZXJ0ZWQgVS1zaGFwZWQsIG9yIE4tc2hhcGVkIHJlbGF0aW9uc2hpcHMgYmV0d2VlbiBhZ3JpY3VsdHVyYWwgYWN0aXZpdGllcyBhbmQgcG9sbHV0aW9uIGxldmVscy4gTGl2ZXN0b2NrIHByb2R1Y3Rpb24gYW5kIHRoZSBleHRlbnNpdmUgdXNlIG9mIHN5bnRoZXRpYyBmZXJ0aWxpc2VycyBhcmUgaWRlbnRpZmllZCBhcyBtYWpvciBjb250cmlidXRvcnMgdG8gZ3JlZW5ob3VzZSBnYXMgKEdIRykgZW1pc3Npb25zLCB3aGlsZSB0aGUgd2lkZXNwcmVhZCB1c2Ugb2YgcGVzdGljaWRlcyBhbmQgaGVyYmljaWRlcyBoYXMgYmVlbiBzaG93biB0byBjYXVzZSBzb2lsIGFuZCB3YXRlciBjb250YW1pbmF0aW9uLiBGdXJ0aGVyIGVudmlyb25tZW50YWwgZGVncmFkYXRpb24gaXMgbGlua2VkIHRvIGRlZm9yZXN0YXRpb24gZHJpdmVuIGJ5IGFncmljdWx0dXJhbCBleHBhbnNpb24sIHdoaWNoIHJlZHVjZXMgY2FyYm9uIHNpbmtzIGFuZCBiaW9kaXZlcnNpdHkuIFRoZSBhZ3JpY3VsdHVyZSBzZWN0b3IncyBkZXBlbmRlbmNlIG9uIGZvc3NpbCBmdWVscyBhbHNvIGV4YWNlcmJhdGVzIGl0cyBHSEcgZW1pc3Npb25zLCB3aGlsZSBpdHMgc2lnbmlmaWNhbnQgZnJlc2h3YXRlciBjb25zdW1wdGlvbiBoZWlnaHRlbnMgY29uY2VybnMgYWJvdXQgd2F0ZXIgc2NhcmNpdHkuIE1vcmVvdmVyLCBzb2lsIGRlZ3JhZGF0aW9uLCBvZnRlbiByZXN1bHRpbmcgZnJvbSBtb25vY3JvcHBpbmcgYW5kIGNvbnZlbnRpb25hbCBmYXJtaW5nIHByYWN0aWNlcywgcHJlc2VudHMgYW4gb25nb2luZyBjaGFsbGVuZ2UuIEhvd2V2ZXIsIHN1c3RhaW5hYmxlIGFncmljdWx0dXJhbCBwcmFjdGljZXMsIHN1Y2ggYXMgYWdyb2ZvcmVzdHJ5LCBjcm9wIHJvdGF0aW9uLCBjb25zZXJ2YXRpb24gdGlsbGFnZSwgYW5kIG9yZ2FuaWMgZmFybWluZywgb2ZmZXIgcHJvbWlzaW5nIHNvbHV0aW9ucyB0byBtaXRpZ2F0ZSB0aGVzZSBlbnZpcm9ubWVudGFsIGltcGFjdHMuIFRoZXNlIHByYWN0aWNlcyBub3Qgb25seSBlbmhhbmNlIHNvaWwgaGVhbHRoIGJ5IHJlZHVjaW5nIGNoZW1pY2FsIGlucHV0cyBidXQgYWxzbyBwcm9tb3RlIGJpb2RpdmVyc2l0eSB3aXRoaW4gZmFybWluZyBzeXN0ZW1zLiBQcmVjaXNpb24gYWdyaWN1bHR1cmUsIG9wdGltaXNhdGlvbiBvZiB3YXRlciwgZmVydGlsaXNlciwgYW5kIHBlc3RpY2lkZSB1c2FnZSwgdGhlIGFkb3B0aW9uIG9mIG5hdGl2ZSBwbGFudCBzcGVjaWVzLCBhbmQgdGhlIGludGVncmF0aW9uIG9mIHJlbmV3YWJsZSBlbmVyZ3kgc291cmNlcyBoYXZlIGJlZW4gaWRlbnRpZmllZCBhcyBrZXkgc3RyYXRlZ2llcyBmb3IgaW1wcm92aW5nIHRoZSBzdXN0YWluYWJpbGl0eSBvZiBhZ3JpY3VsdHVyYWwgb3BlcmF0aW9ucy4gQWRkaXRpb25hbGx5LCBnZW5ldGljIGFkdmFuY2VtZW50cyBpbiBjcm9wIGRldmVsb3BtZW50IG1heSBwbGF5IGEgY3JpdGljYWwgcm9sZSBpbiBhZGRyZXNzaW5nIHRoZSBzZWN0b3IncyBlbnZpcm9ubWVudGFsIGZvb3RwcmludC4gQnkgYWRvcHRpbmcgdGhlc2Ugc3VzdGFpbmFibGUgbWV0aG9kcywgdGhlIGFncmljdWx0dXJlIHNlY3RvciBoYXMgdGhlIHBvdGVudGlhbCB0byBpbmNyZWFzZSBwcm9kdWN0aXZpdHkgd2hpbGUgc2lnbmlmaWNhbnRseSByZWR1Y2luZyBpdHMgZW52aXJvbm1lbnRhbCBpbXBhY3QsIGNvbnRyaWJ1dGluZyB0byB0aGUgb3ZlcmFsbCBnb2FsIG9mIGVjb2xvZ2ljYWwgc3VzdGFpbmFiaWxpdHkuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjU2NTc4L2NpczEyMDMwMiIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9saWJyYXJ5LmFjYWRsb3JlLmNvbS9DSVMvMjAyNC8xMi8zL0NJU18xMi4wM18wMi5wZGYiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMC0yOCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDgiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkNoYWxsZW5nZXMgaW4gU3VzdGFpbmFiaWxpdHkiLCJpc3N1ZSI6Mywidm9sdW1lIjoxMiwiaW1wYWN0RmFjdG9yIjoiMS41OCJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjE3OCwibGFzdFBhZ2UiOjE4OX19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJMaWJyZWxsb3BoIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjowLCJjaXRlZEJ5Q291bnQiOjV9fSwicXVvdGUiOiJJbiBhZGRpdGlvbiwgdGhlIGFncmljdWx0dXJlICBzZWN0b3IgY2FuIHJlZHVjZSBpdHMgZW52aXJvbm1lbnRhbCBmb290cHJpbnQgYnkgcmVkdWNpbmcgdGhlIHVzYWdlIG9mIGZvc3NpbCBmdWVscyBpbiBhZ3JpY3VsdHVyYWwgbWVjaGFuaXphdGlvbi4gIFRoZXJlZm9yZSwgdXNpbmcgc29sYXIsIHdpbmQsIGFuZCBiaW9tYXNzIGVuZXJneSBpbiBmYXJtcyBjYW4gZGVjcmVhc2UgR0hHIGVtaXNzaW9ucyAoS2hhbGVlbCBldCBhbC4sIDIwMjQ7ICBQZW5nIGV0IGFsLiwgMjAyNCkuIEZ1cnRoZXJtb3JlLCBpbmNyZWFzaW5nIGVuZXJneSBlZmZpY2llbmN5IGluIHRoZSBhZ3JpY3VsdHVyZSBzZWN0b3IgY2FuIHJlZHVjZSBvdmVyYWxsIGZ1ZWwgIGNvbnN1bXB0aW9uIGFuZCBlbWlzc2lvbnMsIHdoaWNoIGNvdWxkIGhlbHAgaW4gbWl0aWdhdGluZyBlbnZpcm9ubWVudGFsIHByb2JsZW1zIGluIHRoZSBhZ3JpY3VsdHVyZSBzZWN0b3IuIE1vcmVvdmVyLCB0aGUgbGl0ZXJhdHVyZSBhbHNvIGRpc2N1c3NlZCBtb2Rlcm4gc3VzdGFpbmFibGUgcHJhY3RpY2VzLiBGb3IgaW5zdGFuY2UsIHByZWNpc2lvbiBhZ3JpY3VsdHVyZSB3aXRoIHRoZSBoZWxwIG9mIGRyb25lcyBhbmQgc2Vuc29yIHRlY2hub2xvZ3kgY2FuIG1vbml0b3IgY3JvcCBjb25kaXRpb25zLCBzb2lsIGhlYWx0aCwgYW5kIHdhdGVyLCBmZXJ0aWxpemVycywgYW5kIHBlc3RpY2lkZSBuZWVkcywgd2hpY2ggaGVscHMgdG8gcmVkdWNlIHdhc3RlIGFuZCBlbnZpcm9ubWVudGFsIGltcGFjdCBhbmQgaW1wcm92ZSB5aWVsZCAoRHVzYWRlZXJ1bmdzaWt1bCAmICBOb2YsIDIwMjQpLiBNb3Jlb3ZlciwgZ2VuZXRpYyBhZHZhbmNlbWVudHMgd291bGQgaW5ub3ZhdGUgY3JvcHMgd2l0aCBiZXR0ZXIgcmVzaXN0YW5jZSB0byBwZXN0cywgZGlzZWFzZXMsIGFuZCAgY2xpbWF0aWMgc3RyZXNzZXMsIHdoaWNoIGNhbiByZWR1Y2UgdGhlIG5lZWQgZm9yIGNoZW1pY2FsIGlucHV0cyBjb25zZXF1ZW50bHkgKEh1c2FpbmkgJiBTb2hhaWwsIDIwMjQpLiBUaGUgdXNhZ2UgIG9mIGJpb2xvZ2ljYWxseSBlbmdpbmVlcmVkIGZlcnRpbGl6ZXJzIGFuZCBiaW9mZXJ0aWxpemVycyB3b3VsZCByZWR1Y2UgdGhlIHJlbGlhbmNlIG9uIHN5bnRoZXRpYyBuaXRyb2dlbiBmZXJ0aWxpemVycyAgYW5kIGFsc28gaGVscCBpbiBpbXByb3Zpbmcgc29pbCBoZWFsdGggKEFsemF0ZSBadWx1YWdhIGV0IGFsLiwgMjAyNCkuIFRoZSB1c2FnZSBvZiBiaW9kZWdyYWRhYmxlIG11bGNoIGluIGNyb3AgIHByb3RlY3Rpb24gd291bGQgY29uc2VydmUgbW9pc3R1cmUgYW5kIHJlZHVjZSB0aGUgbmVlZCBmb3IgY2hlbWljYWwgaGVyYmljaWRlcy4gIiwic2NvcmUiOjAuMTg0Nzk5OTk4OTk4NjQxOTcsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6MzAuMTQ1NjI5ODgyODEyNSwibGVmdCI6MTEuOTE2OTUyMTMzMTc4NzExLCJ3aWR0aCI6NzYuNTgyMTA3NTQzOTQ1MzEsImhlaWdodCI6MS41NzgxMzcwNDAxMzgyNDQ2fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6MzEuNTEzOTM1MDg5MTExMzI4LCJsZWZ0IjoxMS45MTY5NTIxMzMxNzg3MTEsIndpZHRoIjo3Ni41OTE4NDI2NTEzNjcxOSwiaGVpZ2h0IjoxLjU3ODEzNzA0MDEzODI0NDZ9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjozMi44ODIyMzI2NjYwMTU2MjUsImxlZnQiOjExLjkxNjk1MjEzMzE3ODcxMSwid2lkdGgiOjc2LjYwNTE2MzU3NDIxODc1LCJoZWlnaHQiOjEuNTc4MTM3MDQwMTM4MjQ0Nn19LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjM0LjIzNjI4MjM0ODYzMjgxLCJsZWZ0IjoxMS45MTY5NTIxMzMxNzg3MTEsIndpZHRoIjo3Ni41MjM3MTIxNTgyMDMxMiwiaGVpZ2h0IjoxLjU3ODEzNzA0MDEzODI0NDZ9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjozNS42MDQ1ODM3NDAyMzQzNzUsImxlZnQiOjExLjkxNjk1MjEzMzE3ODcxMSwid2lkdGgiOjc0LjAyNTA2MjU2MTAzNTE2LCJoZWlnaHQiOjEuNTc4MTM3MDQwMTM4MjQ0Nn19LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjM2Ljk3Mjg4MTMxNzEzODY3LCJsZWZ0IjoxMy42MzAzMTU3ODA2Mzk2NDgsIndpZHRoIjo3NC44MjU4NDM4MTEwMzUxNiwiaGVpZ2h0IjoxLjU3ODEzNzA0MDEzODI0NDZ9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjozOC4zNDExODY1MjM0Mzc1LCJsZWZ0IjoxMS45MTY5NTIxMzMxNzg3MTEsIndpZHRoIjo3Ni41OTU4NTU3MTI4OTA2MiwiaGVpZ2h0IjoxLjU3ODEzNzA0MDEzODI0NDZ9fSx7InBhZ2VOdW1iZXIiOjQsImJib3giOnsidG9wIjozOS43MDk0ODc5MTUwMzkwNiwibGVmdCI6MTEuOTE2OTUyMTMzMTc4NzExLCJ3aWR0aCI6NzYuNTg0MjY2NjYyNTk3NjYsImhlaWdodCI6MS41NzgxMzcwNDAxMzgyNDQ2fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6NDEuMDYzNTMzNzgyOTU4OTg0LCJsZWZ0IjoxMS45MTY5NTIxMzMxNzg3MTEsIndpZHRoIjo3Ni41ODA3MTEzNjQ3NDYxLCJoZWlnaHQiOjEuNTc4MTM3MDQwMTM4MjQ0Nn19LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjQyLjQzMTgzNTE3NDU2MDU1LCJsZWZ0IjoxMS45MTY5NTIxMzMxNzg3MTEsIndpZHRoIjo3Ni42MDAxOTY4MzgzNzg5LCJoZWlnaHQiOjEuNTc4MTM3MDQwMTM4MjQ0Nn19LHsicGFnZU51bWJlciI6NCwiYmJveCI6eyJ0b3AiOjQzLjgwMDEzMjc1MTQ2NDg0NCwibGVmdCI6MTEuOTE2OTUyMTMzMTc4NzExLCJ3aWR0aCI6NzYuNTUwOTE4NTc5MTAxNTYsImhlaWdodCI6MS41NzgxMzcwNDAxMzgyNDQ2fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6NDUuMTY4NDM3OTU3NzYzNjcsImxlZnQiOjExLjkxNjk1MjEzMzE3ODcxMSwid2lkdGgiOjc2LjU1NTgwOTAyMDk5NjEsImhlaWdodCI6MS41NzgxMzcwNDAxMzgyNDQ2fX0seyJwYWdlTnVtYmVyIjo0LCJiYm94Ijp7InRvcCI6NDYuNTM2NzM5MzQ5MzY1MjM0LCJsZWZ0IjoxMS45MTY5NTIxMzMxNzg3MTEsIndpZHRoIjo3Ni41MTg2MjMzNTIwNTA3OCwiaGVpZ2h0IjoxLjU3ODEzNzA0MDEzODI0NDZ9fV19LCJjaHVua0lkIjoiMzYwZjliZWMtOWY4NC00OTVhLTgyMjEtODBlMzAwNjg4YjcyIiwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiMTgyIn19XQ%3D%3D. These inefficiencies necessitate a fundamental shift towards advanced irrigation technologies and sophisticated water management strategies to ensure sustainable agricultural practices (Hashemi et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNleWVkZWjigJBab2hyZWgiLCJmYW1pbHkiOiJIYXNoZW1pIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFiZHVsbGFoIiwiZmFtaWx5IjoiRGFyemnigJBOYWZ0Y2hhbGkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiRmF0ZW1laCIsImZhbWlseSI6IkthcmFuZGlzaCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJILlAuIiwiZmFtaWx5IjoiUml0emVtYSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLYXJpbSIsImZhbWlseSI6IlNvbGFpbWFuaSJ9fV0sImlkcyI6eyJpc3NuIjpbIjAzNzgtMzc3NCIsIjE4NzMtMjI4MyJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMTAxNi9qLmFnd2F0LjIwMjQuMTA5MTEwIiwib3BlbmFsZXgiOiJXNDQwMzQ2MjQ3MyIsImV4YSI6Imh0dHBzOi8vd3d3LnNjaWVuY2VkaXJlY3QuY29tL3NjaWVuY2UvYXJ0aWNsZS9waWkvUzAzNzgzNzc0MjQwMDQ0NjMifSwidGl0bGUiOiJFbmhhbmNpbmcgYWdyaWN1bHR1cmFsIHN1c3RhaW5hYmlsaXR5IHdpdGggd2F0ZXIgYW5kIGNyb3AgbWFuYWdlbWVudCBzdHJhdGVnaWVzIGluIG1vZGVybiBpcnJpZ2F0aW9uIGFuZCBkcmFpbmFnZSBuZXR3b3JrcyIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2ouYWd3YXQuMjAyNC4xMDkxMTAiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2ouYWd3YXQuMjAyNC4xMDkxMTAiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMC0xNiIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkFncmljdWx0dXJhbCBXYXRlciBNYW5hZ2VtZW50Iiwidm9sdW1lIjozMDUsImltcGFjdEZhY3RvciI6IjcuODEifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjoxMDkxMTAsImxhc3RQYWdlIjoxMDkxMTB9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRWxzZXZpZXIgQlYifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjY5LCJjaXRlZEJ5Q291bnQiOjE1fX0sInF1b3RlIjoiQ29uc2VxdWVudGx5LCB0aGUgZGlyZWN0IGVmZmVjdHMgb2YgcG9wdWxhdGlvbiBncm93dGggYW5kIGRpZXRhcnkgY2hhbmdlcyBvbiBkaW1pbmlzaGluZyB3YXRlciByZXNvdXJjZXMsIGFsb25nc2lkZSBjaGFsbGVuZ2VzIHN1Y2ggYXMgY2xpbWF0ZSBjaGFuZ2UsIHNvaWwgZXJvc2lvbiwgZW52aXJvbm1lbnRhbCBwb2xsdXRpb24sIGFuZCByZWR1Y2VkIHJlbmV3YWJsZSB3YXRlciByZXNvdXJjZXMsIG1ha2UgdGhlIHN1c3RhaW5hYmxlIHN1cHBseSBvZiBmb29kIGEgc2lnbmlmaWNhbnQgZnV0dXJlIGNoYWxsZW5nZSAoIFtLYXJhbmRpc2ggZXQgYWwuLCAyMDIyO10oaHR0cHM6Ly93d3cuc2NpZW5jZWRpcmVjdC5jb20vc2NpZW5jZS9hcnRpY2xlL3BpaS9TMDM3ODM3NzQyNDAwNDQ2MyNiaWI0MCkgW0ltYW5pIGV0IGFsLiwgMjAyM10oaHR0cHM6Ly93d3cuc2NpZW5jZWRpcmVjdC5jb20vc2NpZW5jZS9hcnRpY2xlL3BpaS9TMDM3ODM3NzQyNDAwNDQ2MyNiaWIzMSksIFtKYXZhbiBldCBhbC4sIDIwMjRdKGh0dHBzOi8vd3d3LnNjaWVuY2VkaXJlY3QuY29tL3NjaWVuY2UvYXJ0aWNsZS9waWkvUzAzNzgzNzc0MjQwMDQ0NjMjYmliMzQpKS4gQWdyaWN1bHR1cmFsIHN5c3RlbXMsIGFzIHNpZ25pZmljYW50IGNvbnN1bWVycyBvZiB3YXRlciByZXNvdXJjZXMsIGNhbiBoZWxwIGFjaGlldmUgYSBiYWxhbmNlIGJldHdlZW4gd2F0ZXIgc3VwcGx5IGFuZCBkZW1hbmQgYnkgYWRvcHRpbmcgYWR2YW5jZWQgbWFuYWdlbWVudCBzdHJhdGVnaWVzICggW0FiYmFzaSBldCBhbC4sIDIwMTVdKGh0dHBzOi8vd3d3LnNjaWVuY2VkaXJlY3QuY29tL3NjaWVuY2UvYXJ0aWNsZS9waWkvUzAzNzgzNzc0MjQwMDQ0NjMjYmliMiksIFtBcnRoaW5ndG9uIGV0IGFsLiwgMjAxOF0oaHR0cHM6Ly93d3cuc2NpZW5jZWRpcmVjdC5jb20vc2NpZW5jZS9hcnRpY2xlL3BpaS9TMDM3ODM3NzQyNDAwNDQ2MyNiaWI4KSkuIiwic2NvcmUiOjAuMjA2MzAwMDA1MzE2NzM0MzEsImV4dGVybmFsQ2FjaGVJZCI6IjhmMzkwMmQ3LWUzYzctMzQ5Zi05NWRjLTUxOGQ2ZjUyZWU0NCJ9XQ%3D%3D.

## Modern Irrigation Technologies

Modern irrigation technologies mark a transformative evolution from inefficient traditional methods, delivering precise water application that dramatically boosts water use efficiency, slashes energy demands, and elevates crop productivity essential for sustainable agriculture amid escalating water scarcity (Ali et al., 2025)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkF3YWlzIiwiZmFtaWx5IjoiQWxpIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRhamFtdWwiLCJmYW1pbHkiOiJIdXNzYWluIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkF6bGFuIiwiZmFtaWx5IjoiWmFoaWQifX1dLCJpZHMiOnsiaXNzbiI6WyIyNjI0LTc0MDIiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzOTAvYWdyaWVuZ2luZWVyaW5nNzA0MDEwNiIsIm9wZW5hbGV4IjoiVzQ0MDkyMTUwMjEiLCJleGEiOiJodHRwczovL3d3dy5tZHBpLmNvbS8yNjI0LTc0MDIvNy80LzEwNiJ9LCJ0aXRsZSI6IlNtYXJ0IElycmlnYXRpb24gVGVjaG5vbG9naWVzIGFuZCBQcm9zcGVjdHMgZm9yIEVuaGFuY2luZyBXYXRlciBVc2UgRWZmaWNpZW5jeSBmb3IgU3VzdGFpbmFibGUgQWdyaWN1bHR1cmUiLCJhYnN0cmFjdCI6IlJhcGlkIHBvcHVsYXRpb24gZ3Jvd3RoLCByaXNpbmcgZm9vZCBkZW1hbmQsIGFuZCBjbGltYXRlIGNoYW5nZSBoYXZlIGNyZWF0ZWQgc2lnbmlmaWNhbnQgY2hhbGxlbmdlcyB0byBtZWV0IHRoZSB3YXRlciBkZW1hbmRzIGZvciBhZ3JpY3VsdHVyZS4gRWZmZWN0aXZlIGlycmlnYXRpb24gd2F0ZXIgbWFuYWdlbWVudCBpcyBlc3NlbnRpYWwgdG8gYWRkcmVzcyB0aGUgd29ybGTigJlzIHdhdGVyIGNyaXNpcy4gVGhlIHRyYW5zaXRpb24gZnJvbSBjb252ZW50aW9uYWwsIGZyZXF1ZW50bHkgaW5lZmZlY3RpdmUgZ3Jhdml0eS1kcml2ZW4gaXJyaWdhdGlvbnMgdG8gY29udGVtcG9yYXJ5LCBwcmVzc3VyZS1kcml2ZW4gcHJlY2lzaW9uIGlycmlnYXRpb24gbWV0aG9kcyBhcmUgZXhwbG9yZWQgaW4gdGhpcyBhcnRpY2xlLCBhZGRyZXNzaW5nIHRoZSBkaWZmaWN1bHRpZXMgYXNzb2NpYXRlZCB3aXRoIHdhdGVyLWludGVuc2l2ZSBpcnJpZ2F0aW9uLCB0aGUgcG9zc2liaWxpdHkgb2YgdXBkYXRpbmcgY29udmVudGlvbmFsIHRlY2huaXF1ZXMsIGFuZCB0aGUgZGV2ZWxvcG1lbnRzIGluIHNtYXJ0IGFuZCBwcmVjaXNpb24gaXJyaWdhdGlvbiB0ZWNobm9sb2dpZXMuIFRoaXMgc3R1ZHkgY29tcHJlaGVuc2l2ZWx5IGFuYWx5c2VzIHB1Ymxpc2hlZCBsaXRlcmF0dXJlIG9mIDE1MCBhcnRpY2xlcyBmcm9tIHRoZSB5ZWFyIDIwMDUgdG8gMjAyNCwgYmFzZWQgb24gdGl0bGVzLCBhYnN0cmFjdCwgYW5kIGNvbmNsdXNpb25zIHRoYXQgY29udGFpbiBrZXl3b3JkcyBzdWNoIGFzIHByZWNpc2lvbiBpcnJpZ2F0aW9uIHNjaGVkdWxpbmcsIHdhdGVyLXNhdmluZyB0ZWNobm9sb2dpZXMsIGFuZCBzbWFydCBpcnJpZ2F0aW9uIHN5c3RlbXMsIGluIGFkZGl0aW9uIHRvIHByb3ZpZGluZyBwb3RlbnRpYWwgc29sdXRpb25zIHRvIGFjaGlldmUgc3VzdGFpbmFibGUgZGV2ZWxvcG1lbnQgZ29hbHMgYW5kIHNtYXJ0IGFncmljdWx0dXJhbCBwcm9kdWN0aW9uIHN5c3RlbXMuIE1vcmVvdmVyLCBpdCBleHBsb3JlcyB0aGUgZnVuZGFtZW50YWxzIGFuZCBwcm9jZXNzZXMgb2Ygc21hcnQgaXJyaWdhdGlvbiwgc3VjaCBhcyBvcGVuLSBhbmQgY2xvc2VkLWxvb3AgY29udHJvbCwgcHJlY2lzaW9uIG1vbml0b3JpbmcgYW5kIGNvbnRyb2wgc3lzdGVtcywgYW5kIHNtYXJ0IG1vbml0b3JpbmcgbWV0aG9kcyBiYXNlZCBvbiBzb2lsIGRhdGEsIHBsYW50IHdhdGVyIHN0YXR1cywgd2VhdGhlciBkYXRhLCByZW1vdGUgc2Vuc2luZywgYW5kIHBhcnRpY2lwYXRvcnkgaXJyaWdhdGlvbiBtYW5hZ2VtZW50LiBMaWtld2lzZSwgdG8gZW1waGFzaXplIHRoZSBwb3RlbnRpYWwgb2YgdGhlc2UgdGVjaG5vbG9naWVzIGZvciBhIG1vcmUgc3VzdGFpbmFibGUgYWdyaWN1bHR1cmFsIGZ1dHVyZSwgc2V2ZXJhbCBzbWFydCB0ZWNobmlxdWVzLCBpbmNsdWRpbmcgSW9ULCB3aXJlbGVzcyBzZW5zb3IgbmV0d29ya3MsIGRlZXAgbGVhcm5pbmcsIGFuZCBmdXp6eSBsb2dpYywgYW5kIHRoZWlyIGVmZmVjdHMgb24gY3JvcCBwZXJmb3JtYW5jZSBhbmQgd2F0ZXIgY29uc2VydmF0aW9uIGFjcm9zcyB2YXJpb3VzIGNyb3BzIGFyZSBkaXNjdXNzZWQuIFRoZSByZXZpZXcgY29uY2x1ZGVzIGJ5IHN1bW1hcml6aW5nIHRoZSBsaW1pdGF0aW9ucyBhbmQgY2hhbGxlbmdlcyBvZiBpbXBsZW1lbnRpbmcgcHJlY2lzaW9uIGlycmlnYXRpb24gc3lzdGVtcyBhbmQgQUkgaW4gYWdyaWN1bHR1cmUgYWxvbmcgd2l0aCBoaWdobGlnaHRpbmcgdGhlIHJlbGF0aW9uc2hpcCBvZiBhZG9wdGluZyBwcmVjaXNpb24gaXJyaWdhdGlvbiBhbmQgdWx0aW1hdGVseSBhY2hpZXZpbmcgdmFyaW91cyBzdXN0YWluYWJsZSBkZXZlbG9wbWVudCBnb2FscyAoU0RHcykuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjMzOTAvYWdyaWVuZ2luZWVyaW5nNzA0MDEwNiIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjMzOTAvYWdyaWVuZ2luZWVyaW5nNzA0MDEwNiIsImlzc3VlZERhdGUiOiIyMDI1LTA0LTA0IiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiQWdyaUVuZ2luZWVyaW5nIiwiaXNzdWUiOjQsInZvbHVtZSI6NywiaW1wYWN0RmFjdG9yIjoiMy4zMiJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjEwNiwibGFzdFBhZ2UiOjEwNn19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJNdWx0aWRpc2NpcGxpbmFyeSBEaWdpdGFsIFB1Ymxpc2hpbmcgSW5zdGl0dXRlIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjowLCJjaXRlZEJ5Q291bnQiOjV9fSwicXVvdGUiOiJFZmZlY3RpdmUgaXJyaWdhdGlvbiB3YXRlciBtYW5hZ2VtZW50IGlzIGVzc2VudGlhbCB0byBhZGRyZXNzIHRoZSB3b3JsZOKAmXMgd2F0ZXIgY3Jpc2lzLiBUaGUgdHJhbnNpdGlvbiBmcm9tIGNvbnZlbnRpb25hbCwgZnJlcXVlbnRseSBpbmVmZmVjdGl2ZSBncmF2aXR5LWRyaXZlbiBpcnJpZ2F0aW9ucyB0byBjb250ZW1wb3JhcnksIHByZXNzdXJlLWRyaXZlbiBwcmVjaXNpb24gaXJyaWdhdGlvbiBtZXRob2RzIGFyZSBleHBsb3JlZCBpbiB0aGlzIGFydGljbGUsIGFkZHJlc3NpbmcgdGhlIGRpZmZpY3VsdGllcyBhc3NvY2lhdGVkIHdpdGggd2F0ZXItaW50ZW5zaXZlIGlycmlnYXRpb24sIHRoZSBwb3NzaWJpbGl0eSBvZiB1cGRhdGluZyBjb252ZW50aW9uYWwgdGVjaG5pcXVlcywgYW5kIHRoZSBkZXZlbG9wbWVudHMgaW4gc21hcnQgYW5kIHByZWNpc2lvbiBpcnJpZ2F0aW9uIHRlY2hub2xvZ2llcy4gVGhpcyBzdHVkeSBjb21wcmVoZW5zaXZlbHkgYW5hbHlzZXMgcHVibGlzaGVkIGxpdGVyYXR1cmUgb2YgMTUwIGFydGljbGVzIGZyb20gdGhlIHllYXIgMjAwNSB0byAyMDI0LCBiYXNlZCBvbiB0aXRsZXMsIGFic3RyYWN0LCBhbmQgY29uY2x1c2lvbnMgdGhhdCBjb250YWluIGtleXdvcmRzIHN1Y2ggYXMgcHJlY2lzaW9uIGlycmlnYXRpb24gc2NoZWR1bGluZywgd2F0ZXItc2F2aW5nIHRlY2hub2xvZ2llcywgYW5kIHNtYXJ0IGlycmlnYXRpb24gc3lzdGVtcywgaW4gYWRkaXRpb24gdG8gcHJvdmlkaW5nIHBvdGVudGlhbCBzb2x1dGlvbnMgdG8gYWNoaWV2ZSBzdXN0YWluYWJsZSBkZXZlbG9wbWVudCBnb2FscyBhbmQgc21hcnQgYWdyaWN1bHR1cmFsIHByb2R1Y3Rpb24gc3lzdGVtcy4iLCJzY29yZSI6MC4yMjE3OTk5OTk0NzU0NzkxMywiZXh0ZXJuYWxDYWNoZUlkIjoiNDc4NGQzMWMtMzA3My0zYmRjLWIwYTgtYzVkMGYxNWVlNDE4In1d. Unlike surface irrigation's high losses from evaporation, runoff, and percolation—which consume ~70% of global freshwater—advanced systems like drip, micro, sprinkler, pivot, and subsurface drip target water directly to plant roots, curbing wastage by up to 50-90% while minimizing environmental degradation such as soil erosion and salinization (Evans & Sadler, 2008)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjozLCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlJvYmVydCBHLiIsImZhbWlseSI6IkV2YW5zIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkUuIEouIiwiZmFtaWx5IjoiU2FkbGVyIn19XSwiaWRzIjp7Imlzc24iOlsiMDA0My0xMzk3IiwiMTk0NC03OTczIl0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDI5LzIwMDd3cjAwNjIwMCIsIm1hZyI6IjE1MTcyNTY0OTYiLCJvcGVuYWxleCI6IlcxNTE3MjU2NDk2IiwiZXhhIjoiaHR0cHM6Ly9hZ3VwdWJzLm9ubGluZWxpYnJhcnkud2lsZXkuY29tL2RvaS9mdWxsLzEwLjEwMjkvMjAwN1dSMDA2MjAwIn0sInRpdGxlIjoiTWV0aG9kcyBhbmQgdGVjaG5vbG9naWVzIHRvIGltcHJvdmUgZWZmaWNpZW5jeSBvZiB3YXRlciB1c2UiLCJhYnN0cmFjdCI6IlRoZSBjb21wZXRpdGlvbiBmb3IgZXhpc3RpbmcgZnJlc2h3YXRlciBzdXBwbGllcyB3aWxsIHJlcXVpcmUgYSBwYXJhZGlnbWF0aWMgc2hpZnQgZnJvbSBtYXhpbWl6aW5nIHByb2R1Y3Rpdml0eSBwZXIgdW5pdCBvZiBsYW5kIGFyZWEgdG8gbWF4aW1pemluZyBwcm9kdWN0aXZpdHkgcGVyIHVuaXQgb2Ygd2F0ZXIgY29uc3VtZWQuIFRoaXMgc2hpZnQgd2lsbCwgaW4gdHVybiwgZGVtYW5kIGJyb2FkIHN5c3RlbXMgYXBwcm9hY2hlcyB0aGF0IHBoeXNpY2FsbHkgYW5kIGJpb2xvZ2ljYWxseSBvcHRpbWl6ZSBpcnJpZ2F0aW9uIHJlbGF0aXZlIHRvIHdhdGVyIGRlbGl2ZXJ5IGFuZCBhcHBsaWNhdGlvbiBzY2hlbWVzLCByYWluZmFsbCwgY3JpdGljYWwgZ3Jvd3RoIHN0YWdlcywgc29pbCBmZXJ0aWxpdHksIGxvY2F0aW9uLCBhbmQgd2VhdGhlci4gV2F0ZXIgY2FuIGJlIGNvbnNlcnZlZCBhdCBhIHdhdGVyc2hlZCBvciByZWdpb25hbCBsZXZlbCBmb3Igb3RoZXIgdXNlcyBvbmx5IGlmIGV2YXBvcmF0aW9uLCB0cmFuc3BpcmF0aW9uLCBvciBib3RoIGFyZSByZWR1Y2VkIGFuZCB1bnJlY292ZXJhYmxlIGxvc3NlcyB0byB1bnVzYWJsZSBzaW5rcyBhcmUgbWluaW1pemVkIChlLmcuLCBzYWx0eSBncm91bmR3YXRlciBvciBvY2VhbnMpLiBBZ3JpY3VsdHVyYWwgYWR2YW5jZXMgd2lsbCBpbmNsdWRlIGltcGxlbWVudGF0aW9uIG9mIGNyb3AgbG9jYXRpb24gc3RyYXRlZ2llcywgY29udmVyc2lvbiB0byBjcm9wcyB3aXRoIGhpZ2hlciBlY29ub21pYyB2YWx1ZSBvciBwcm9kdWN0aXZpdHkgcGVyIHVuaXQgb2Ygd2F0ZXIgY29uc3VtZWQsIGFuZCBhZG9wdGlvbiBvZiBhbHRlcm5hdGUgZHJvdWdodOKAkHRvbGVyYW50IGNyb3BzLiBFbWVyZ2luZyBjb21wdXRlcml6ZWQgR1BT4oCQYmFzZWQgcHJlY2lzaW9uIGlycmlnYXRpb24gdGVjaG5vbG9naWVzIGZvciBzZWxm4oCQcHJvcGVsbGVkIHNwcmlua2xlcnMgYW5kIG1pY3JvaXJyaWdhdGlvbiBzeXN0ZW1zIHdpbGwgZW5hYmxlIGdyb3dlcnMgdG8gYXBwbHkgd2F0ZXIgYW5kIGFncm9jaGVtaWNhbHMgbW9yZSBwcmVjaXNlbHkgYW5kIHNpdGUgc3BlY2lmaWNhbGx5IHRvIG1hdGNoIHNvaWwgYW5kIHBsYW50IHN0YXR1cyBhbmQgbmVlZHMgYXMgcHJvdmlkZWQgYnkgd2lyZWxlc3Mgc2Vuc29yIG5ldHdvcmtzLiBBZ3JpY3VsdHVyYWxpc3RzIHdpbGwgbmVlZCB0byBleGVyY2lzZSBmbGV4aWJpbGl0eSBpbiBtYW5hZ2luZyB0aGUgcmF0ZSwgZnJlcXVlbmN5LCBhbmQgZHVyYXRpb24gb2Ygd2F0ZXIgc3VwcGxpZXMgdG8gc3VjY2Vzc2Z1bGx5IGFsbG9jYXRlIGxpbWl0ZWQgd2F0ZXIgYW5kIG90aGVyIGlucHV0cyB0byBjcm9wcy4gVGhlIG1vc3QgZWZmZWN0aXZlIG1lYW5zIHRvIGNvbnNlcnZlIHdhdGVyIGFwcGVhcnMgdG8gYmUgdGhyb3VnaCBjYXJlZnVsbHkgbWFuYWdlZCBkZWZpY2l0IGlycmlnYXRpb24gc3RyYXRlZ2llcyB0aGF0IGFyZSBzdXBwb3J0ZWQgYnkgYWR2YW5jZWQgaXJyaWdhdGlvbiBzeXN0ZW0gYW5kIGZsZXhpYmxlLCBzdGF0ZeKAkG9m4oCQdGhl4oCQYXJ0IHdhdGVyIGRlbGl2ZXJ5IHN5c3RlbXMuIE5vbmFncmljdWx0dXJhbCB3YXRlciB1c2VycyB3aWxsIG5lZWQgdG8gZXhlcmNpc2UgcGF0aWVuY2UgYXMgdG9vbHMgcmVmbGVjdGluZyB0aGUgcGFyYWRpZ21hdGljIHNoaWZ0IGFyZSBhY3R1YWxpemVkLiBCb3RoIGdyb3VwcyB3aWxsIG5lZWQgdG8gY29vcGVyYXRlIGFuZCBjb21wcm9taXNlIGFzIHRoZXkgcHJhY3RpY2UgbW9yZSBjb25zZXJ2YXRpdmUgYXBwcm9hY2hlcyB0byBmcmVzaHdhdGVyIGNvbnN1bXB0aW9uLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDI5LzIwMDd3cjAwNjIwMCIsImlzc3VlZERhdGUiOiIyMDA4LTA3LTAxIiwiYWNjZXNzZWREYXRlIjoiMjAyNi0wMSIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiV2F0ZXIgUmVzb3VyY2VzIFJlc2VhcmNoIiwiaXNzdWUiOjcsInZvbHVtZSI6NDQsImltcGFjdEZhY3RvciI6IjYuNDAifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiV2lsZXkifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjY4LCJjaXRlZEJ5Q291bnQiOjQ1N319LCJxdW90ZSI6IlxcWzFcXF0gVGhlIGNvbXBldGl0aW9uIGZvciBleGlzdGluZyBmcmVzaHdhdGVyIHN1cHBsaWVzIHdpbGwgcmVxdWlyZSBhIHBhcmFkaWdtYXRpYyBzaGlmdCBmcm9tIG1heGltaXppbmcgcHJvZHVjdGl2aXR5IHBlciB1bml0IG9mIGxhbmQgYXJlYSB0byBtYXhpbWl6aW5nIHByb2R1Y3Rpdml0eSBwZXIgdW5pdCBvZiB3YXRlciBjb25zdW1lZC4gVGhpcyBzaGlmdCB3aWxsLCBpbiB0dXJuLCBkZW1hbmQgYnJvYWQgc3lzdGVtcyBhcHByb2FjaGVzIHRoYXQgcGh5c2ljYWxseSBhbmQgYmlvbG9naWNhbGx5IG9wdGltaXplIGlycmlnYXRpb24gcmVsYXRpdmUgdG8gd2F0ZXIgZGVsaXZlcnkgYW5kIGFwcGxpY2F0aW9uIHNjaGVtZXMsIHJhaW5mYWxsLCBjcml0aWNhbCBncm93dGggc3RhZ2VzLCBzb2lsIGZlcnRpbGl0eSwgbG9jYXRpb24sIGFuZCB3ZWF0aGVyLiBXYXRlciBjYW4gYmUgY29uc2VydmVkIGF0IGEgd2F0ZXJzaGVkIG9yIHJlZ2lvbmFsIGxldmVsIGZvciBvdGhlciB1c2VzIG9ubHkgaWYgZXZhcG9yYXRpb24sIHRyYW5zcGlyYXRpb24sIG9yIGJvdGggYXJlIHJlZHVjZWQgYW5kIHVucmVjb3ZlcmFibGUgbG9zc2VzIHRvIHVudXNhYmxlIiwic2NvcmUiOjAuMTU0NDAwMDA1OTM2NjIyNjJ9XQ%3D%3D. Precision irrigation, integrating targeted delivery to individual plants or zones, exemplifies this shift, optimizing resource use without compromising yields and addressing critical barriers like groundwater depletion (Jumintono et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikp1bWludG9ubyIsImZhbWlseSI6Ikp1bWludG9ubyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJOdXJuYXdhdHkiLCJmYW1pbHkiOiJOdXJuYXdhdHkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTnVncm9obyBUcmlzbnUiLCJmYW1pbHkiOiJCcmF0YSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJEaW5pIiwiZmFtaWx5IjoiUmFobWlhdGkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSGFkaSIsImZhbWlseSI6IlN1c2lsbyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBLiBBLiIsImZhbWlseSI6Ikh1c3NlbiJ9fV0sImlkcyI6eyJpc3NuIjpbIjE0MjktNzQyNiIsIjIwODMtNDUzNSJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMjQ0MjUvandsZC4yMDI0LjE1MTc5OCIsIm9wZW5hbGV4IjoiVzQ0MDU0NzcxMDAifSwidGl0bGUiOiJDb21wYXJhdGl2ZSBhbmFseXNpcyBvZiBwcmVjaXNpb24sIGRlZmljaXQsIGFuZCBkcmlwIGlycmlnYXRpb24gZm9yIHdhdGVyIGVmZmljaWVuY3kgaW4gSXJhcSIsImFic3RyYWN0IjoiU3VzdGFpbmFibGUgaXJyaWdhdGlvbiBwcmFjdGljZXMgYXJlIGNyaXRpY2FsIGZvciBhZGRyZXNzaW5nIHRoZSBjaGFsbGVuZ2VzIG9mIHdhdGVyIHNjYXJjaXR5IGFuZCBmb29kIGluc2VjdXJpdHkgaW4gdGhlIGZhY2Ugb2YgY2xpbWF0ZSBjaGFuZ2UgYW5kIGdyb3dpbmcgZ2xvYmFsIGZvb2QgZGVtYW5kLiBUaGlzIHN0dWR5IGludmVzdGlnYXRlZCB0aGUgcG90ZW50aWFsIG9mIHByZWNpc2lvbiBpcnJpZ2F0aW9uIChQSSksIGRlZmljaXQgaXJyaWdhdGlvbiAoREkpLCBhbmQgZHJpcCBpcnJpZ2F0aW9uIChEUkkpIHRlY2huaXF1ZXMgdG8gZW5oYW5jZSBjcm9wIHByb2R1Y3Rpdml0eSBhbmQgd2F0ZXIgdXNlIGVmZmljaWVuY3kgKFdVRSkgY29tcGFyZWQgdG8gY29udmVudGlvbmFsIGZsb29kIGlycmlnYXRpb24gKENGSSkgaW4gdGhyZWUgZGlzdGluY3QgYWdyb2Vjb2xvZ2ljYWwgem9uZXMgb2YgSXJhcS4gRmllbGQgZXhwZXJpbWVudHMgd2VyZSBjb25kdWN0ZWQgdXNpbmcgYSByYW5kb21pc2VkIGNvbXBsZXRlIGJsb2NrIGRlc2lnbiB3aXRoIHdoZWF0LCBtYWl6ZSwgYW5kIHJpY2UgY3JvcHMuIFRoZSByZXN1bHRzIGRlbW9uc3RyYXRlZCB0aGF0IFBJIHNpZ25pZmljYW50bHkgaW5jcmVhc2VkIGNyb3AgZ3Jvd3RoIHBhcmFtZXRlcnMsIGdyYWluIHlpZWxkLCBhbmQgV1VFIGFjcm9zcyBhbGwgem9uZXMsIHdpdGggeWllbGQgaW1wcm92ZW1lbnRzIG9mIDMz4oCTMzglIGFuZCBXVUUgaW5jcmVhc2VzIG9mIDQ24oCTNTElIGluIGNvbnRyYXN0IHdpdGggQ0ZJLiBUaGUgREkgYW5kIERSSSB0cmVhdG1lbnRzIGFsc28gb3V0cGVyZm9ybWVkIENGSSwgYWxiZWl0IHRvIGEgbGVzc2VyIGV4dGVudC4gUmVtb3RlIHNlbnNpbmctZGVyaXZlZCB2ZWdldGF0aW9uIGluZGljZXMgc3Ryb25nbHkgY29ycmVsYXRlZCB3aXRoIGNyb3AgZ3Jvd3RoIHBhcmFtZXRlcnMgYW5kIHlpZWxkLCB3aGlsZSBoeWRyb2xvZ2ljYWwgbW9kZWxsaW5nIHJldmVhbGVkIHJlZHVjZWQgZXZhcG90cmFuc3BpcmF0aW9uIGFuZCBzdXJmYWNlIHJ1bm9mZiB1bmRlciB0aGUgUEkgdHJlYXRtZW50LiBUaGUgc3VzdGFpbmFibGUgaXJyaWdhdGlvbiBwcmFjdGljZXMgcmVzdWx0ZWQgaW4gc3Vic3RhbnRpYWwgd2F0ZXIgc2F2aW5ncyBvZiAyMOKAkzMwJSBjb21wYXJlZCB0byBDRkkuIFRoZXNlIGZpbmRpbmdzIGhpZ2hsaWdodCB0aGUgaW1wb3J0YW5jZSBvZiBhZG9wdGluZyBlZmZpY2llbnQgaXJyaWdhdGlvbiB0ZWNobmlxdWVzLCBhbG9uZyB3aXRoIGEgaG9saXN0aWMgYXBwcm9hY2ggZW5jb21wYXNzaW5nIHRlY2hub2xvZ2ljYWwgaW5ub3ZhdGlvbnMsIGNhcGFjaXR5IGJ1aWxkaW5nLCBhbmQgc3Rha2Vob2xkZXIgZW5nYWdlbWVudCwgdG8gcHJvbW90ZSB3YXRlci0gZWZmaWNpZW50IGFncmljdWx0dXJlIGFuZCBlbnN1cmUgZm9vZCBzZWN1cml0eSBpbiB3YXRlci1zY2FyY2UgcmVnaW9ucy4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjQ0MjUvandsZC4yMDI0LjE1MTc5OCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjI0NDI1L2p3bGQuMjAyNC4xNTE3OTgiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMi0xNCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkpvdXJuYWwgb2YgV2F0ZXIgYW5kIExhbmQgRGV2ZWxvcG1lbnQiLCJpbXBhY3RGYWN0b3IiOiIxLjQ2In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTMwLCJsYXN0UGFnZSI6MTM2fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkRlIEdydXl0ZXIgT3BlbiJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MzIsImNpdGVkQnlDb3VudCI6MX19LCJxdW90ZSI6IkluIHRoaXMgY29udGV4dCwgdGhlIGFkb3B0aW9uIG9mICBzdXN0YWluYWJsZSBpcnJpZ2F0aW9uIHByYWN0aWNlcyBoYXMgZW1lcmdlZCBhcyBhIGNyaXRpY2FsIHN0cmF0ZWd5ICBmb3IgZW5oYW5jaW5nIGZvb2Qgc2VjdXJpdHkgd2hpbGUgcHJvbW90aW5nIHdhdGVyIGNvbnNlcnZhdGlvbiAgaW4gYWdyaWN1bHR1cmFsIGxhbmRzY2FwZXMuICBJcnJpZ2F0aW9uIHBsYXlzIGEgdml0YWwgcm9sZSBpbiBnbG9iYWwgZm9vZCBwcm9kdWN0aW9uLCB3aXRoICBpcnJpZ2F0ZWQgYWdyaWN1bHR1cmUgYWNjb3VudGluZyBmb3IgYXBwcm94aW1hdGVseSA0MCUgb2YgdGhlICBmb29kIHN1cHBseSAoV3JhY2hpZW4gZGUsIFNjaHVsdHogYW5kIEdvbGksIDIwMjE7IFNjaG1pdHQsIFJvc2EgIGFuZCBEYWlseSwgMjAyMikuIEhvd2V2ZXIsIHRyYWRpdGlvbmFsIGlycmlnYXRpb24gbWV0aG9kcywgc3VjaCBhcyAgZmxvb2QgYW5kIGZ1cnJvdyBpcnJpZ2F0aW9uLCBhcmUgb2Z0ZW4gaW5lZmZpY2llbnQsIGxlYWRpbmcgdG8gIHN1YnN0YW50aWFsIHdhdGVyIGxvc3NlcyB0aHJvdWdoIGV2YXBvcmF0aW9uLCBydW5vZmYsIGFuZCBkZWVwICBwZXJjb2xhdGlvbiAoQW1ib21zYSwgMjAyMCkuIE1vcmVvdmVyLCB0aGUgZXhjZXNzaXZlIGFwcGxpY2EtdGlvbiBvZiB3YXRlciBjYW4gcmVzdWx0IGluIHNvaWwgc2FsaW5pc2F0aW9uLCBudXRyaWVudCBsZWFjaGluZywgYW5kICBncm91bmR3YXRlciBkZXBsZXRpb24sIGZ1cnRoZXIgY29tcHJvbWlzaW5nIHRoZSBsb25nLXRlcm0gIHN1c3RhaW5hYmlsaXR5IG9mIGFncmljdWx0dXJhbCBzeXN0ZW1zIChZaW4gZXQgYWwuLCAyMDIyKS4gVG8gYWRkcmVzcyAgdGhlc2UgY2hhbGxlbmdlcywgcmVzZWFyY2hlcnMgYW5kIHByYWN0aXRpb25lcnMgaGF2ZSBiZWVuIGV4cGxvci1pbmcgaW5ub3ZhdGl2ZSBpcnJpZ2F0aW9uIHRlY2huaXF1ZXMgYW5kIHdhdGVyIG1hbmFnZW1lbnQgIHN0cmF0ZWdpZXMgdGhhdCBjYW4gb3B0aW1pc2Ugd2F0ZXIgdXNlIGVmZmljaWVuY3kgKFdVRSkgd2hpbGUgIG1haW50YWluaW5nIG9yIGVuaGFuY2luZyBjcm9wIHByb2R1Y3Rpdml0eS4gIFByZWNpc2lvbiBpcnJpZ2F0aW9uIChQSSkgb2ZmZXJzIGEgcHJvbWlzaW5nIHNvbHV0aW9uLCAgaW52b2x2aW5nIHRoZSB0YXJnZXRlZCBhcHBsaWNhdGlvbiBvZiB3YXRlciBzcGVjaWZpY2FsbHkgdG8gdGhlICBuZWVkcyBvZiBpbmRpdmlkdWFsIHBsYW50cyBvciBjcm9wIHpvbmVzIChaaW5rZXJuYWdlbCBldCBhbC4sIDIwMjApLiAgIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyMi44ODY3MzU5MTYxMzc2OTUsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzQ2MjIxOTIzODI4MTI1LCJoZWlnaHQiOjEuNDUwNTQxMDE5NDM5Njk3M319LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI0LjM1MTEyMzgwOTgxNDQ1MywibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTI3MTYwNjQ0NSwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI1LjcyNDk4NzAzMDAyOTI5NywibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTY1MzA3NjE3MiwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI3LjA5ODg1MDI1MDI0NDE0LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjE2LjQxODE2MTM5MjIxMTkxNCwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI4LjQ3MjcwOTY1NTc2MTcyLCJsZWZ0IjoxMi4zODA4ODAzNTU4MzQ5NjEsIndpZHRoIjozNy4wMTIxODQxNDMwNjY0MDYsImhlaWdodCI6MS4zNjAwMTQ3OTYyNTcwMTl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyOS44NDY1NzI4NzU5NzY1NjIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQwNjQ4NjUxMTIzMDUsImhlaWdodCI6MS4zNjAwMTY3MDM2MDU2NTE5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzEuMjE0MDM2OTQxNTI4MzIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQ0ODU2MjYyMjA3MDMsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzIuNTg3OTAyMDY5MDkxOCwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDYwNTc4OTE4NDU3LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMzLjk2MTc2MTQ3NDYwOTM3NSwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDA0ODQ2MTkxNDA2MjUsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzUuMzM1NjI0Njk0ODI0MjIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzM5NzQ0NTY3ODcxMDk0LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjM2LjcwOTQ4NzkxNTAzOTA2LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjAwMTU5MDcyODc1OTc2NiwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC4wODMzNDczMjA1NTY2NCwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTY1MzA3NjE3MiwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOS40NTcyMTA1NDA3NzE0ODQsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQzMDQ4MDk1NzAzMTI1LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQwLjc0MDU0NzE4MDE3NTc4LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjM0NTQ1NTE2OTY3NzczNCwiaGVpZ2h0IjoxLjQ1MDU0MjgwNzU3OTA0MDV9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Mi4yMDQ5MzMxNjY1MDM5MDYsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMDAzMjUwMTIyMDcwMzEsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDMuNTc4Nzk2Mzg2NzE4NzUsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzM3OTIxMTQyNTc4MTI1LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQ0Ljg2MjEzMzAyNjEyMzA1LCJsZWZ0Ijo5LjA0NzYwMTY5OTgyOTEwMiwid2lkdGgiOjQwLjM0NTMxNDAyNTg3ODkwNiwiaGVpZ2h0IjoxLjQ1MDU0MjgwNzU3OTA0MDV9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ni4zMjY1MTkwMTI0NTExNywibGVmdCI6OS4wNDc2MDE2OTk4MjkxMDIsIndpZHRoIjoyNy45Njc0NDcyODA4ODM3OSwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ny43MDAzODIyMzI2NjYwMTYsImxlZnQiOjEyLjM4MDg3ODQ0ODQ4NjMyOCwid2lkdGgiOjM3LjAzNDc4NjIyNDM2NTIzNCwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0OS4wNzQyNDU0NTI4ODA4NiwibGVmdCI6OS4wNDc2MDE2OTk4MjkxMDIsIndpZHRoIjo0MC4zNDA2NDQ4MzY0MjU3OCwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1MC4zNTc1ODIwOTIyODUxNTYsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzQ0ODUyNDQ3NTA5NzY2LCJoZWlnaHQiOjEuNDUwNTQyODA3NTc5MDQwNX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEzMSJ9fV0%3D. These innovations, supported by empirical evidence, compel widespread adoption to secure food production against climate pressures and resource limits

### Table 2: Critical Limitations of Traditional Irrigation

| Limitation Category | Specific Challenge | Environmental/Economic Impact |
| --- | --- | --- |
| Water Loss | Evaporation | Significant freshwater wastage |
| Water Loss | Runoff | Loss of nutrients and water |
| Distribution | Nonuniformity | Suboptimal crop yields |
| Resource Use | Deep Percolation | Leaching of essential nutrients |
| Soil Health | Erosion | Long-term land degradation |
| Resource Use | Energy Overuse | High operational costs/emissions |
| Productivity | Suboptimal Yields | Inability to meet food demand |
| Global Impact | Freshwater Usage | Consumes ~70% of global freshwater |

### Drip and Micro-Irrigation Systems

Drip and micro-irrigation systems deliver water directly to the plant root zone through low-pressure emitters, tubes, or micro-sprayers, achieving application efficiencies of 90-95% by minimizing evaporation, runoff, and deep percolation compared to traditional methods (Plaščak et al., 2021)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikl2YW4iLCJmYW1pbHkiOiJQbGHFocSNYWsifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTWxhZGVuIiwiZmFtaWx5IjoiSnVyacWhacSHIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkRvcmlqYW4iLCJmYW1pbHkiOiJSYWRvxI1haiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNaWxhbiIsImZhbWlseSI6IlZ1amnEhyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJEb21hZ29qIiwiZmFtaWx5IjoiWmltbWVyIn19XSwiaWRzIjp7Imlzc24iOlsiMTg0Ni02MTY4IiwiMTg0OC01NTg4Il0sImlzYm4iOltdLCJkb2kiOiIxMC4zMTgwMy90Zy0yMDIxMDQxNjEwMzUwMCIsIm1hZyI6IjMyMDk2MTA5NDIiLCJvcGVuYWxleCI6IlczMjA5NjEwOTQyIiwib3BlbkFjY2Vzc0lkIjoiMTMxNTMwNzAyNTc0OTE2NTU5NzgifSwidGl0bGUiOiJBbiBPdmVydmlldyBvZiBQcmVjaXNpb24gSXJyaWdhdGlvbiBTeXN0ZW1zIFVzZWQgaW4gQWdyaWN1bHR1cmUiLCJhYnN0cmFjdCI6IlRoZSBpbnRyb2R1Y3Rpb24gb2YgcHJlY2lzaW9uIGFncmljdWx0dXJlIGluY3JlYXNlZCB0aGUgZWZmaWNpZW5jeSBvZiBwbGFudCBwcm9kdWN0aW9uLCB3aGlsZSBzaW11bHRhbmVvdXNseSByZWR1Y2luZyB0aGUgcHJvZHVjdGlvbiBjb3N0LiBQcmVjaXNpb24gaXJyaWdhdGlvbiBjYW4gYmUgY29uc2lkZXJlZCBhcyB0aGUgY29tYmluYXRpb24gb2Ygc2Vuc29ycywgY29tcHV0ZXIgc29mdHdhcmUgYW5kIGlycmlnYXRpb24gc3lzdGVtcy4gUHJlY2lzaW9uIGlycmlnYXRpb24gaGFzIHJlZHVjZWQgd2F0ZXIgY29uc3VtcHRpb24gYW5kIGluY3JlYXNlZCB5aWVsZHMsIGFuZCB0aHVzIGluY3JlYXNlZCBlY29ub21pYyBwcm9maXRzLiBUaGUgZGV2ZWxvcG1lbnQgb2YgbmV3IGNyb3AgbW9uaXRvcmluZyB0ZWNobm9sb2dpZXMgaW4gcHJlY2lzaW9uIGlycmlnYXRpb24gaGFzIGJlZW4gbWFkZSBwb3NzaWJsZSBieSB0aGUgaW1hZ2luZyBhbmQgYW5hbHlzaXMgb2YgcmVhbC10aW1lIGNyb3AgY29uZGl0aW9uIGRhdGEuIFRoZSBhaW0gb2YgdGhpcyBzdHVkeSB3YXMgdG8gZGVzY3JpYmUgdGhlIHByZXNlbnQgc3RhdGUgYW5kIHBvc3NpYmlsaXRpZXMgb2YgcHJlY2lzaW9uIGlycmlnYXRpb24gaW4gcHJhY3RpY2UgaW4gdGhlIEVVIGFuZCBDcm9hdGlhLiBBbiBvdmVydmlldyBvZiB0aGUgY3VycmVudCBwcmVjaXNpb24gaXJyaWdhdGlvbiB0ZWNobm9sb2dpZXMsIGFzIHdlbGwgYXMgaXRzIGFkYXB0aXZlIG1hbmFnZW1lbnQgdG8gdGhlIGRlY2lzaW9uLW1ha2luZyBpbiBhZ3JpY3VsdHVyYWwgd2F0ZXIgbWFuYWdlbWVudCwgcmVwcmVzZW50cyBhIGZ1bmRhbWVudGFsIGJhc2lzIGZvciBmdXR1cmUgcHJhY3RpY2FsIHN0dWRpZXMgaW4gcHJlY2lzaW9uIGlycmlnYXRpb24uIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjMxODAzL3RnLTIwMjEwNDE2MTAzNTAwIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2hyY2FrLnNyY2UuaHIvZmlsZS8zODQ3MDAiLCJpc3N1ZWREYXRlIjoiMjAyMS0xMS0wMSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IlRlaG5pxI1raSBnbGFzbmlrIiwiaXNzdWUiOjQsInZvbHVtZSI6MTUsImltcGFjdEZhY3RvciI6IjEuMTAifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjo1NDYsImxhc3RQYWdlIjo1NTN9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiQ3JvYXRpYW4gRGFpcnkgVW5pb24ifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjM1LCJjaXRlZEJ5Q291bnQiOjh9fSwicXVvdGUiOiJUaGUgYWR2YW50YWdlcyAgb2YgdGhpcyBhcHByb2FjaCBhcmU6IGlycmlnYXRpb24gb2YgYSBzbWFsbGVyIGFyZWEsIG1pbmltYWwgIHdhdGVyIGV2YXBvcmF0aW9uIGZyb20gdGhlIHNvaWwgc3VyZmFjZSwgcmVkdWNlZCB3ZWVkICBwcmVzZW5jZSwgYW5kIHVuaWZvcm0gd2F0ZXIgYXBwbGljYXRpb24gaW4gdGhlIHJvb3Qgem9uZSBvZiAgdGhlIHBsYW50LiBBIHNwZWNpYWwgYWR2YW50YWdlIG9mIG1pY3JvLWlycmlnYXRpb24gaXMgdGhlICBhZGRpdGlvbiBvZiBhIHNtYWxsZXIgYW1vdW50IG9mIHdhdGVyIGF0IHNob3J0IGlycmlnYXRpb24gIGludGVydmFscy4gVGhpcyBwcm92aWRlcyB0aGUgYWJpbGl0eSB0byBtYWludGFpbiBzb2lsIG1vaXN0dXJlICB3aXRoIGEgY2VydGFpbiB3YXRlciBkZWZpY2l0IGJlbG93IHRoZSBjYXBhY2l0eSBvZiB0aGUgZmllbGQuIEl0ICBjYW4gYmUgbWFuYWdlZCB0aHJvdWdob3V0IHRoZSBlbnRpcmUgc2Vhc29uIG9yIG9ubHkgaXRzIHBhcnQsICB0aHVzIGFjaGlldmluZyBncmVhdGVyIGlycmlnYXRpb24gZWZmaWNpZW5jeSBhbmQgcmVkdWNpbmcgIGlycmlnYXRpb24gY29zdHMuIFRoZSBlZmZpY2llbmN5IG9mIG1pY3JvLWlycmlnYXRpb24gY2FuIGJlICBoaWdoZXIgdGhhbiA5MCAlIFs0MF0uIFdhdGVyIGxvc3NlcyBpbiBtaWNyby1pcnJpZ2F0aW9uIG1haW5seSBvY2N1ciBieSBldmFwb3JhdGlvbiBvZiB3YXRlciBmcm9tIHRoZSBzb2lsIHN1cmZhY2UgYW5kIG91dGZsb3cgb2YgIHdhdGVyIGludG8gdGhlIGRyYWluYWdlLiBFdmFwb3JhdGlvbiBsb3NzZXMgYXJlIHNtYWxsIGR1ZSB0byAgdGhlIGxpbWl0ZWQgaXJyaWdhdGlvbiBhcmVhIGFuZCBubyBzdGFuZGluZyB3YXRlciBvbiB0aGUgc29pbCAgc3VyZmFjZS4gVGhlIGNhdXNlcyBvZiBwb3NzaWJseSB1bmV2ZW4gaXJyaWdhdGlvbiBpbiBtaWNyby1zeXN0ZW1zIGFyZSB0aGUgbGVuZ3RoIG9mIHRoZSBwaXBlLCBwcmVzc3VyZSBvc2NpbGxhdGlvbnMgYW5kICBjbG9nZ2luZyBvZiB0aGUgYXBwbGljYXRvciBkdXJpbmcgaXRzIG9wZXJhdGlvbi4gVGhlIG1pY3JvLWlycmlnYXRpb24gc3lzdGVtIGhhcyBhIGhpZ2hlciBwb3RlbnRpYWwgZm9yIHdpZGVzcHJlYWQgIGltcGxlbWVudGF0aW9uIGNvbXBhcmVkIHRvIG90aGVyIHN5c3RlbXMuIE1hbmFnZW1lbnQgaXMgIGVhc3kgYW5kIGlzIHVzdWFsbHkgYXV0b21hdGVkIGJhc2VkIG9uIHRpbWUsIHNvaWwgbW9pc3R1cmUsICBvciBzdXJyb3VuZGluZyB0ZW1wZXJhdHVyZSBbNDBdLiBBdXRob3JzIFs0MV0gdXNlZCBhIHN5c3RlbSBmb3IgbW9uaXRvcmluZyB0aGUgbW9pc3R1cmUgaW4gdGhlIHJvb3Qgem9uZSBpbiB2aXRpY3VsdHVyZSBhbmQgbWVhc3VyZWQgc29pbCBtb2lzdHVyZSBpbiAgYSBjbG9zZWQtbG9vcCBzeXN0ZW0uICIsInNjb3JlIjowLjE3NTYwMDAwNzE3NjM5OTIzLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MywiYmJveCI6eyJ0b3AiOjU5LjQ2Nzk5NDY4OTk0MTQwNiwibGVmdCI6OS4yNTQwNDU0ODY0NTAxOTUsIndpZHRoIjozOS45ODcwMjIzOTk5MDIzNDQsImhlaWdodCI6MS40MDA4NDUxNzAwMjEwNTcxfX0seyJwYWdlTnVtYmVyIjozLCJiYm94Ijp7InRvcCI6NjAuOTE1MTA3NzI3MDUwNzgsImxlZnQiOjkuMjUyNDA5OTM0OTk3NTU5LCJ3aWR0aCI6MzkuOTg5MzIyNjYyMzUzNTE2LCJoZWlnaHQiOjEuNDAwODQ1MTcwMDIxMDU3MX19LHsicGFnZU51bWJlciI6MywiYmJveCI6eyJ0b3AiOjYyLjM2OTgxMjAxMTcxODc1LCJsZWZ0Ijo5LjI1MjQwNzA3Mzk3NDYxLCJ3aWR0aCI6MzkuOTkwNDc0NzAwOTI3NzM0LCJoZWlnaHQiOjEuNDAwODQ1MTcwMDIxMDU3MX19LHsicGFnZU51bWJlciI6MywiYmJveCI6eyJ0b3AiOjYzLjgyNDUxNjI5NjM4NjcyLCJsZWZ0Ijo5LjI1MjQwODAyNzY0ODkyNiwid2lkdGgiOjM5Ljk4NjUzNDExODY1MjM0NCwiaGVpZ2h0IjoxLjQwMDg0NTE3MDAyMTA1NzF9fSx7InBhZ2VOdW1iZXIiOjMsImJib3giOnsidG9wIjo2NS4yNzE2MzY5NjI4OTA2MiwibGVmdCI6OS4yNTI0MDgwMjc2NDg5MjYsIndpZHRoIjozOS45OTA2MzQ5MTgyMTI4OSwiaGVpZ2h0IjoxLjQwMDg0NTE3MDAyMTA1NzF9fSx7InBhZ2VOdW1iZXIiOjMsImJib3giOnsidG9wIjo2Ni43MjYzNDEyNDc1NTg2LCJsZWZ0Ijo5LjI1NDA0NzM5Mzc5ODgyOCwid2lkdGgiOjM5Ljk4ODgzODE5NTgwMDc4LCJoZWlnaHQiOjEuNDAwODQ1MTcwMDIxMDU3MX19LHsicGFnZU51bWJlciI6MywiYmJveCI6eyJ0b3AiOjY4LjE4MTA0NTUzMjIyNjU2LCJsZWZ0Ijo5LjI1NDA0ODM0NzQ3MzE0NSwid2lkdGgiOjM5Ljk4NTU0OTkyNjc1NzgxLCJoZWlnaHQiOjEuNDAwODQ1MTcwMDIxMDU3MX19LHsicGFnZU51bWJlciI6MywiYmJveCI6eyJ0b3AiOjY5LjYyODE1MDkzOTk0MTQsImxlZnQiOjkuMjU0MDQ4MzQ3NDczMTQ1LCJ3aWR0aCI6MzkuOTkxNjE1Mjk1NDEwMTU2LCJoZWlnaHQiOjEuNDAwODQ1MTcwMDIxMDU3MX19LHsicGFnZU51bWJlciI6MywiYmJveCI6eyJ0b3AiOjcxLjA4Mjg1NTIyNDYwOTM4LCJsZWZ0Ijo5LjI1NTY4Mjk0NTI1MTQ2NSwid2lkdGgiOjM5Ljk5MDQ1OTQ0MjEzODY3LCJoZWlnaHQiOjEuNDAwODQ1MTcwMDIxMDU3MX19LHsicGFnZU51bWJlciI6MywiYmJveCI6eyJ0b3AiOjcyLjUzNzU1OTUwOTI3NzM0LCJsZWZ0Ijo5LjI1NzMxODQ5NjcwNDEwMiwid2lkdGgiOjM5Ljk4ODY2NjUzNDQyMzgzLCJoZWlnaHQiOjEuNDAwODQ1MTcwMDIxMDU3MX19LHsicGFnZU51bWJlciI6MywiYmJveCI6eyJ0b3AiOjczLjk4NDY3MjU0NjM4NjcyLCJsZWZ0Ijo5LjI1NTY4NDg1MjYwMDA5OCwid2lkdGgiOjM5Ljk4ODY2MjcxOTcyNjU2LCJoZWlnaHQiOjEuNDAwODQ1MTcwMDIxMDU3MX19LHsicGFnZU51bWJlciI6MywiYmJveCI6eyJ0b3AiOjc1LjQzOTM3NjgzMTA1NDY5LCJsZWZ0Ijo5LjI1NDA0OTMwMTE0NzQ2MSwid2lkdGgiOjE1LjE0OTgyNDE0MjQ1NjA1NSwiaGVpZ2h0IjoxLjQwMDg0NTE3MDAyMTA1NzF9fSx7InBhZ2VOdW1iZXIiOjMsImJib3giOnsidG9wIjo3Ni44ODY0ODk4NjgxNjQwNiwibGVmdCI6OS4yNTA3NzYyOTA4OTM1NTUsIndpZHRoIjozOS45OTE5NDMzNTkzNzUsImhlaWdodCI6MTUuOTI1MTEyNzI0MzA0Mn19LHsicGFnZU51bWJlciI6MywiYmJveCI6eyJ0b3AiOjcuMTk3Mjc0MjA4MDY4ODQ4LCJsZWZ0Ijo1Ni4yNDM4NzM1OTYxOTE0MDYsIndpZHRoIjozNy4yMTUwOTkzMzQ3MTY4LCJoZWlnaHQiOjEuNDAwODQ0MjE2MzQ2NzQwN319LHsicGFnZU51bWJlciI6MywiYmJveCI6eyJ0b3AiOjguNjQ0Mzg5MTUyNTI2ODU1LCJsZWZ0Ijo1My40Njg2Njk4OTEzNTc0Miwid2lkdGgiOjM5Ljk4ODY1ODkwNTAyOTMsImhlaWdodCI6MS40MDA4NDQyMTYzNDY3NDA3fX0seyJwYWdlTnVtYmVyIjozLCJiYm94Ijp7InRvcCI6MTAuMDk4MzQ2NzEwMjA1MDc4LCJsZWZ0Ijo1My40NjcwMjE5NDIxMzg2Nywid2lkdGgiOjM5Ljk5MjA3Njg3Mzc3OTMsImhlaWdodCI6MS40MDE1OTE1MzkzODI5MzQ2fX1dfSwiY2h1bmtJZCI6IjQ3OTkyYjY5LWEzYmItNDYxMC04Y2FlLWU3YmY5ZmUwODY5ZiIsImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjU0OSJ9fV0%3D.

Principles of Operation

These systems operate on the principle of low-volume, high-frequency water application. Drip systems use buried or surface tubing with inline or online emitters spaced according to crop needs, while micro-irrigation employs finer nozzles for spray or mist. Filters, pressure regulators, and valves ensure uniform distribution, with fertigation capabilities integrating nutrients for enhanced uptake (Hla & Scherer, 2003)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjozLCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkF1bmcgSy4iLCJmYW1pbHkiOiJIbGEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiVGhvbWFzIEYuIiwiZmFtaWx5IjoiU2NoZXJlciJ9fV0sImlkcyI6eyJpc3NuIjpbXSwiaXNibiI6W10sIm1hZyI6Ijc2MjM3MjI3MCIsIm9wZW5hbGV4IjoiVzc2MjM3MjI3MCIsImV4YSI6Imh0dHBzOi8vaXJyaWdhdGlvbnRvb2xib3guY29tL1JlZmVyZW5jZURvY3VtZW50cy9FeHRlbnNpb24vTm9ydGhEYWtvdGEvYWUxMjQzLnBkZiJ9LCJ0aXRsZSI6IkludHJvZHVjdGlvbiB0byBNaWNyby1pcnJpZ2F0aW9uIiwidXJsIjoiaHR0cDovL2hkbC5oYW5kbGUubmV0LzEwMzY1LzUxMDgiLCJyZXNvdXJjZVVybCI6Imh0dHA6Ly9oZGwuaGFuZGxlLm5ldC8xMDM2NS81MTA4IiwiaXNzdWVkRGF0ZSI6IjIwMDMtMDEtMDEiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTEwIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJORFNVIFJlcG9zaXRvcnkgKE5vcnRoIERha290YSBTdGF0ZSBVbml2ZXJzaXR5KSIsImltcGFjdEZhY3RvciI6IjAuMDUifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiTm9ydGggRGFrb3RhIFN0YXRlIFVuaXZlcnNpdHkifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjAsImNpdGVkQnlDb3VudCI6OH19LCJxdW90ZSI6IkFFLTEyNDNcbkludHJvZHVjdGlvbiB0b1xuTWljcm8taXJyaWdhdGlvblxuQXVuZyBLLiBIbGEsIEFyZWEgRXh0ZW5zaW9uIElycmlnYXRpb24gU3BlY2lhbGlzdFxuVGhvbWFzIEYuIFNjaGVyZXIsIEV4dGVuc2lvbiBBZ3JpY3VsdHVyYWwgRW5naW5lZXJcbk5vcnRoIERha290YSBTdGF0ZSBVbml2ZXJzaXR5XG5GYXJnbywgTm9ydGggRGFrb3RhIDU4MTA1XG5NQVJDSCAyMDAzXG4yXG5NaWNyby1pcnJpZ2F0aW9uIHJlZmVycyB0b1xubG93LXByZXNzdXJlIGlycmlnYXRpb24gc3lzdGVtc1xudGhhdCBzcHJheSwgbWlzdCwgc3ByaW5rbGUgb3IgZHJpcC5cblRoZSB3YXRlciBkaXNjaGFyZ2UgcGF0dGVybnNcbmRpZmZlciBiZWNhdXNlIGVtaXNzaW9uIGRldmljZXNcbmFyZSBkZXNpZ25lZCBmb3Igc3BlY2lmaWNcbmFwcGxpY2F0aW9ucyBkdWUgdG8gYWdyb25vbWljXG5vciBob3J0aWN1bHR1cmFsIHJlcXVpcmVtZW50cy5cbk1pY3JvLWlycmlnYXRpb24gY29tcG9uZW50c1xuaW5jbHVkZSBwaXBlcywgdHViZXMsIHdhdGVyXG5lbWl0dGluZyBkZXZpY2VzLCBmbG93IGNvbnRyb2xcbmVxdWlwbWVudCwgaW5zdGFsbGF0aW9uIHRvb2xzLFxuZml0dGluZ3MgYW5kIGFjY2Vzc29yaWVzLiIsInNjb3JlIjowLjI4OTQwMDAxMTMwMTA0MDY1fV0%3D.

Advantages and Efficiency Gains

- Water savings: Reduce consumption by 50-90% versus surface irrigation, critical as agriculture uses ~70% of global freshwater (Daraz et al., 2025; Jumintono et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlVtYXIiLCJmYW1pbHkiOiJEYXJheiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiLFoHRlZmFuIiwiZmFtaWx5IjoiQm9qbmVjIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllvdW5hcyIsImZhbWlseSI6IktoYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMDc3LTA0NzIiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzOTAvYWdyaWN1bHR1cmUxNTA1MDU1NCIsIm9wZW5hbGV4IjoiVzQ0MDgyMTk4MTEiLCJleGEiOiJodHRwczovL3d3dy5tZHBpLmNvbS8yMDc3LTA0NzIvMTUvNS81NTQifSwidGl0bGUiOiJFbmVyZ3ktRWZmaWNpZW50IFNtYXJ0IElycmlnYXRpb24gVGVjaG5vbG9naWVzOiBBIFBhdGh3YXkgdG8gV2F0ZXIgYW5kIEVuZXJneSBTdXN0YWluYWJpbGl0eSBpbiBBZ3JpY3VsdHVyZSIsImFic3RyYWN0IjoiVGhlIGFncmljdWx0dXJhbCBzZWN0b3IgZmFjZXMgY2hhbGxlbmdlcyBzdWNoIGFzIHdhdGVyIHNjYXJjaXR5LCBlbmVyZ3kgaW5lZmZpY2llbmN5LCBhbmQgZGVjbGluaW5nIHByb2R1Y3Rpdml0eSwgcGFydGljdWxhcmx5IGluIGFyaWQgcmVnaW9ucy4gVHJhZGl0aW9uYWwgaXJyaWdhdGlvbiBtZXRob2RzIGNvbnRyaWJ1dGUgdG8gcmVzb3VyY2UgZGVwbGV0aW9uIGFuZCBlbnZpcm9ubWVudGFsIGltcGFjdHMuIFNvbGFyLXBvd2VyZWQgc21hcnQgaXJyaWdhdGlvbiBzeXN0ZW1zIGludGVncmF0ZSBwcmVjaXNpb24gaXJyaWdhdGlvbiB3aXRoIHJlbmV3YWJsZSBlbmVyZ3ksIGltcHJvdmluZyB3YXRlciB1c2UgYW5kIHByb2R1Y3Rpdml0eS4gSW4gUGFraXN0YW4sIHdoZXJlIGFncmljdWx0dXJlIGNvbnRyaWJ1dGVzIDE5JSBvZiBncm9zcyBkb21lc3RpYyBwcm9kdWN0IGFuZCBlbXBsb3lzIDQwJSBvZiB0aGUgd29ya2ZvcmNlLCB0aGVzZSBjaGFsbGVuZ2VzIGFyZSBzZXZlcmUsIGVzcGVjaWFsbHkgaW4gd2F0ZXItc2NhcmNlIGFyZWFzIGxpa2UgdGhlIENob2xpc3RhbiBEZXNlcnQuIFRoaXMgc3R1ZHkgZXhhbWluZXMgdGhlIGltcGFjdCBvZiBzb2xhci1wb3dlcmVkIHNtYXJ0IGlycmlnYXRpb24gb24gYWdyaWN1bHR1cmFsIHByb2R1Y3Rpdml0eSwgd2F0ZXIgY29uc2VydmF0aW9uLCBhbmQgZW5lcmd5IGVmZmljaWVuY3kgaW4gdGhlIENob2xpc3RhbiBEZXNlcnQuIFVzaW5nIGEgcXVhbnRpdGF0aXZlIGNyb3NzLXNlY3Rpb25hbCBkZXNpZ24sIGRhdGEgd2VyZSBjb2xsZWN0ZWQgZnJvbSAzODQgZmFybWVycyB2aWEgc3RydWN0dXJlZCBxdWVzdGlvbm5haXJlcy4gU3RhdGlzdGljYWwgYW5hbHlzZXMsIGluY2x1ZGluZyBtdWx0aXBsZSBsaW5lYXIgcmVncmVzc2lvbiwgcGFpcmVkIHNhbXBsZSB0LXRlc3RzLCBhbmQgU3RydWN0dXJhbCBFcXVhdGlvbiBNb2RlbGluZyAoU0VNKSwgd2VyZSBjb25kdWN0ZWQuIEZpbmRpbmdzIHNob3cgc2lnbmlmaWNhbnQgaW1wcm92ZW1lbnRzIGluIGNyb3AgeWllbGQgKGZyb20gMy4wIHRvIDQuOCB0b25zL2hlY3RhcmUpIGFuZCByZWR1Y3Rpb25zIGluIHdhdGVyIGFuZCBlbmVyZ3kgY29uc3VtcHRpb24uIFJlZ3Jlc3Npb24gYW5hbHlzaXMgaGlnaGxpZ2h0ZWQgc3Ryb25nIHBvc2l0aXZlIGVmZmVjdHMgb24geWllbGQgYW5kIGVmZmljaWVuY3ksIHdoaWxlIFNFTSBjb25maXJtZWQgcmVkdWNlZCBlbnZpcm9ubWVudGFsIGltcGFjdCBhbmQgb3BlcmF0aW9uYWwgY29zdHMuIFRoZSBzdHVkeSBjb25jbHVkZXMgdGhhdCBzb2xhci1wb3dlcmVkIGlycmlnYXRpb24gZW5oYW5jZXMgcHJvZHVjdGl2aXR5LCBjb25zZXJ2ZXMgcmVzb3VyY2VzLCBhbmQgcHJvbW90ZXMgc3VzdGFpbmFiaWxpdHkuIFBvbGljeW1ha2VycyBzaG91bGQgcHJvdmlkZSBmaW5hbmNpYWwgaW5jZW50aXZlcywgaW52ZXN0IGluIHJlbmV3YWJsZSBpbmZyYXN0cnVjdHVyZSwgYW5kIGltcGxlbWVudCB0cmFpbmluZyBwcm9ncmFtcyB0byBzdXBwb3J0IGFkb3B0aW9uLiBDb2xsYWJvcmF0aXZlIGVmZm9ydHMgYXJlIGVzc2VudGlhbCBmb3Igc3VzdGFpbmFibGUgYWdyaWN1bHR1cmUgaW4gYXJpZCByZWdpb25zLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJpc3N1ZWREYXRlIjoiMjAyNS0wMy0wNSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkFncmljdWx0dXJlIiwiaXNzdWUiOjUsInZvbHVtZSI6MTUsImltcGFjdEZhY3RvciI6IjUuMjYifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjo1NTQsImxhc3RQYWdlIjo1NTR9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiTXVsdGlkaXNjaXBsaW5hcnkgRGlnaXRhbCBQdWJsaXNoaW5nIEluc3RpdHV0ZSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NDgsImNpdGVkQnlDb3VudCI6Mn19LCJxdW90ZSI6IjEuIEludHJvZHVjdGlvbiBUaGUgYWdyaWN1bHR1cmFsIHNlY3RvciwgYSBjb3JuZXJzdG9uZSBvZiBnbG9iYWwgZm9vZCBzZWN1cml0eSBhbmQgZWNvbm9taWMgZGV2ZWxvcG1lbnQsIGZhY2VzIG1vdW50aW5nIGNoYWxsZW5nZXMgaW4gdGhlIDIxc3QgY2VudHVyeS4gQ2xpbWF0ZSBjaGFuZ2UsIHJhcGlkIHBvcHVsYXRpb24gZ3Jvd3RoLCBhbmQgZHdpbmRsaW5nIG5hdHVyYWwgcmVzb3VyY2VzIGhhdmUgaW50ZW5zaWZpZWQgdGhlIG5lZWQgZm9yIHN1c3RhaW5hYmxlIGFncmljdWx0dXJhbCBwcmFjdGljZXMuIEFtb25nIHRoZXNlIGNoYWxsZW5nZXMsIHdhdGVyIHNjYXJjaXR5IGFuZCBlbmVyZ3kgaW5lZmZpY2llbmN5IHN0YW5kIG91dCBhcyBjcml0aWNhbCBiYXJyaWVycyB0byBwcm9kdWN0aXZpdHkgYW5kIHN1c3RhaW5hYmlsaXR5LiBHbG9iYWxseSwgYWdyaWN1bHR1cmUgYWNjb3VudHMgZm9yIGFwcHJveGltYXRlbHkgNzAlIG9mIGZyZXNod2F0ZXIgd2l0aGRyYXdhbHMgYW5kIGlzIG9uZSBvZiB0aGUgbGFyZ2VzdCBjb25zdW1lcnMgb2YgZW5lcmd5IGluIG1hbnkgcmVnaW9ucyBbIDEsIDJdLiBUcmFkaXRpb25hbCBpcnJpZ2F0aW9uIHByYWN0aWNlcywgb2Z0ZW4gaW5lZmZpY2llbnQgYW5kIHJlc291cmNlLWludGVuc2l2ZSwgZXhhY2VyYmF0ZSB0aGVzZSBpc3N1ZXMsIGxlYWRpbmcgdG8gd2F0ZXIgd2FzdGFnZSwgZW5lcmd5IG92ZXJ1c2UsIGFuZCByZWR1Y2VkIGNyb3AgeWllbGRzLiJ9LHsib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikp1bWludG9ubyIsImZhbWlseSI6Ikp1bWludG9ubyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJOdXJuYXdhdHkiLCJmYW1pbHkiOiJOdXJuYXdhdHkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTnVncm9obyBUcmlzbnUiLCJmYW1pbHkiOiJCcmF0YSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJEaW5pIiwiZmFtaWx5IjoiUmFobWlhdGkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSGFkaSIsImZhbWlseSI6IlN1c2lsbyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBLiBBLiIsImZhbWlseSI6Ikh1c3NlbiJ9fV0sImlkcyI6eyJpc3NuIjpbIjE0MjktNzQyNiIsIjIwODMtNDUzNSJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMjQ0MjUvandsZC4yMDI0LjE1MTc5OCIsIm9wZW5hbGV4IjoiVzQ0MDU0NzcxMDAifSwidGl0bGUiOiJDb21wYXJhdGl2ZSBhbmFseXNpcyBvZiBwcmVjaXNpb24sIGRlZmljaXQsIGFuZCBkcmlwIGlycmlnYXRpb24gZm9yIHdhdGVyIGVmZmljaWVuY3kgaW4gSXJhcSIsImFic3RyYWN0IjoiU3VzdGFpbmFibGUgaXJyaWdhdGlvbiBwcmFjdGljZXMgYXJlIGNyaXRpY2FsIGZvciBhZGRyZXNzaW5nIHRoZSBjaGFsbGVuZ2VzIG9mIHdhdGVyIHNjYXJjaXR5IGFuZCBmb29kIGluc2VjdXJpdHkgaW4gdGhlIGZhY2Ugb2YgY2xpbWF0ZSBjaGFuZ2UgYW5kIGdyb3dpbmcgZ2xvYmFsIGZvb2QgZGVtYW5kLiBUaGlzIHN0dWR5IGludmVzdGlnYXRlZCB0aGUgcG90ZW50aWFsIG9mIHByZWNpc2lvbiBpcnJpZ2F0aW9uIChQSSksIGRlZmljaXQgaXJyaWdhdGlvbiAoREkpLCBhbmQgZHJpcCBpcnJpZ2F0aW9uIChEUkkpIHRlY2huaXF1ZXMgdG8gZW5oYW5jZSBjcm9wIHByb2R1Y3Rpdml0eSBhbmQgd2F0ZXIgdXNlIGVmZmljaWVuY3kgKFdVRSkgY29tcGFyZWQgdG8gY29udmVudGlvbmFsIGZsb29kIGlycmlnYXRpb24gKENGSSkgaW4gdGhyZWUgZGlzdGluY3QgYWdyb2Vjb2xvZ2ljYWwgem9uZXMgb2YgSXJhcS4gRmllbGQgZXhwZXJpbWVudHMgd2VyZSBjb25kdWN0ZWQgdXNpbmcgYSByYW5kb21pc2VkIGNvbXBsZXRlIGJsb2NrIGRlc2lnbiB3aXRoIHdoZWF0LCBtYWl6ZSwgYW5kIHJpY2UgY3JvcHMuIFRoZSByZXN1bHRzIGRlbW9uc3RyYXRlZCB0aGF0IFBJIHNpZ25pZmljYW50bHkgaW5jcmVhc2VkIGNyb3AgZ3Jvd3RoIHBhcmFtZXRlcnMsIGdyYWluIHlpZWxkLCBhbmQgV1VFIGFjcm9zcyBhbGwgem9uZXMsIHdpdGggeWllbGQgaW1wcm92ZW1lbnRzIG9mIDMz4oCTMzglIGFuZCBXVUUgaW5jcmVhc2VzIG9mIDQ24oCTNTElIGluIGNvbnRyYXN0IHdpdGggQ0ZJLiBUaGUgREkgYW5kIERSSSB0cmVhdG1lbnRzIGFsc28gb3V0cGVyZm9ybWVkIENGSSwgYWxiZWl0IHRvIGEgbGVzc2VyIGV4dGVudC4gUmVtb3RlIHNlbnNpbmctZGVyaXZlZCB2ZWdldGF0aW9uIGluZGljZXMgc3Ryb25nbHkgY29ycmVsYXRlZCB3aXRoIGNyb3AgZ3Jvd3RoIHBhcmFtZXRlcnMgYW5kIHlpZWxkLCB3aGlsZSBoeWRyb2xvZ2ljYWwgbW9kZWxsaW5nIHJldmVhbGVkIHJlZHVjZWQgZXZhcG90cmFuc3BpcmF0aW9uIGFuZCBzdXJmYWNlIHJ1bm9mZiB1bmRlciB0aGUgUEkgdHJlYXRtZW50LiBUaGUgc3VzdGFpbmFibGUgaXJyaWdhdGlvbiBwcmFjdGljZXMgcmVzdWx0ZWQgaW4gc3Vic3RhbnRpYWwgd2F0ZXIgc2F2aW5ncyBvZiAyMOKAkzMwJSBjb21wYXJlZCB0byBDRkkuIFRoZXNlIGZpbmRpbmdzIGhpZ2hsaWdodCB0aGUgaW1wb3J0YW5jZSBvZiBhZG9wdGluZyBlZmZpY2llbnQgaXJyaWdhdGlvbiB0ZWNobmlxdWVzLCBhbG9uZyB3aXRoIGEgaG9saXN0aWMgYXBwcm9hY2ggZW5jb21wYXNzaW5nIHRlY2hub2xvZ2ljYWwgaW5ub3ZhdGlvbnMsIGNhcGFjaXR5IGJ1aWxkaW5nLCBhbmQgc3Rha2Vob2xkZXIgZW5nYWdlbWVudCwgdG8gcHJvbW90ZSB3YXRlci0gZWZmaWNpZW50IGFncmljdWx0dXJlIGFuZCBlbnN1cmUgZm9vZCBzZWN1cml0eSBpbiB3YXRlci1zY2FyY2UgcmVnaW9ucy4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjQ0MjUvandsZC4yMDI0LjE1MTc5OCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjI0NDI1L2p3bGQuMjAyNC4xNTE3OTgiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMi0xNCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkpvdXJuYWwgb2YgV2F0ZXIgYW5kIExhbmQgRGV2ZWxvcG1lbnQiLCJpbXBhY3RGYWN0b3IiOiIxLjQ2In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTMwLCJsYXN0UGFnZSI6MTM2fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkRlIEdydXl0ZXIgT3BlbiJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MzIsImNpdGVkQnlDb3VudCI6MX19LCJxdW90ZSI6IkluIHRoaXMgY29udGV4dCwgdGhlIGFkb3B0aW9uIG9mICBzdXN0YWluYWJsZSBpcnJpZ2F0aW9uIHByYWN0aWNlcyBoYXMgZW1lcmdlZCBhcyBhIGNyaXRpY2FsIHN0cmF0ZWd5ICBmb3IgZW5oYW5jaW5nIGZvb2Qgc2VjdXJpdHkgd2hpbGUgcHJvbW90aW5nIHdhdGVyIGNvbnNlcnZhdGlvbiAgaW4gYWdyaWN1bHR1cmFsIGxhbmRzY2FwZXMuICBJcnJpZ2F0aW9uIHBsYXlzIGEgdml0YWwgcm9sZSBpbiBnbG9iYWwgZm9vZCBwcm9kdWN0aW9uLCB3aXRoICBpcnJpZ2F0ZWQgYWdyaWN1bHR1cmUgYWNjb3VudGluZyBmb3IgYXBwcm94aW1hdGVseSA0MCUgb2YgdGhlICBmb29kIHN1cHBseSAoV3JhY2hpZW4gZGUsIFNjaHVsdHogYW5kIEdvbGksIDIwMjE7IFNjaG1pdHQsIFJvc2EgIGFuZCBEYWlseSwgMjAyMikuIEhvd2V2ZXIsIHRyYWRpdGlvbmFsIGlycmlnYXRpb24gbWV0aG9kcywgc3VjaCBhcyAgZmxvb2QgYW5kIGZ1cnJvdyBpcnJpZ2F0aW9uLCBhcmUgb2Z0ZW4gaW5lZmZpY2llbnQsIGxlYWRpbmcgdG8gIHN1YnN0YW50aWFsIHdhdGVyIGxvc3NlcyB0aHJvdWdoIGV2YXBvcmF0aW9uLCBydW5vZmYsIGFuZCBkZWVwICBwZXJjb2xhdGlvbiAoQW1ib21zYSwgMjAyMCkuIE1vcmVvdmVyLCB0aGUgZXhjZXNzaXZlIGFwcGxpY2EtdGlvbiBvZiB3YXRlciBjYW4gcmVzdWx0IGluIHNvaWwgc2FsaW5pc2F0aW9uLCBudXRyaWVudCBsZWFjaGluZywgYW5kICBncm91bmR3YXRlciBkZXBsZXRpb24sIGZ1cnRoZXIgY29tcHJvbWlzaW5nIHRoZSBsb25nLXRlcm0gIHN1c3RhaW5hYmlsaXR5IG9mIGFncmljdWx0dXJhbCBzeXN0ZW1zIChZaW4gZXQgYWwuLCAyMDIyKS4gVG8gYWRkcmVzcyAgdGhlc2UgY2hhbGxlbmdlcywgcmVzZWFyY2hlcnMgYW5kIHByYWN0aXRpb25lcnMgaGF2ZSBiZWVuIGV4cGxvci1pbmcgaW5ub3ZhdGl2ZSBpcnJpZ2F0aW9uIHRlY2huaXF1ZXMgYW5kIHdhdGVyIG1hbmFnZW1lbnQgIHN0cmF0ZWdpZXMgdGhhdCBjYW4gb3B0aW1pc2Ugd2F0ZXIgdXNlIGVmZmljaWVuY3kgKFdVRSkgd2hpbGUgIG1haW50YWluaW5nIG9yIGVuaGFuY2luZyBjcm9wIHByb2R1Y3Rpdml0eS4gIFByZWNpc2lvbiBpcnJpZ2F0aW9uIChQSSkgb2ZmZXJzIGEgcHJvbWlzaW5nIHNvbHV0aW9uLCAgaW52b2x2aW5nIHRoZSB0YXJnZXRlZCBhcHBsaWNhdGlvbiBvZiB3YXRlciBzcGVjaWZpY2FsbHkgdG8gdGhlICBuZWVkcyBvZiBpbmRpdmlkdWFsIHBsYW50cyBvciBjcm9wIHpvbmVzIChaaW5rZXJuYWdlbCBldCBhbC4sIDIwMjApLiAgIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyMi44ODY3MzU5MTYxMzc2OTUsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzQ2MjIxOTIzODI4MTI1LCJoZWlnaHQiOjEuNDUwNTQxMDE5NDM5Njk3M319LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI0LjM1MTEyMzgwOTgxNDQ1MywibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTI3MTYwNjQ0NSwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI1LjcyNDk4NzAzMDAyOTI5NywibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTY1MzA3NjE3MiwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI3LjA5ODg1MDI1MDI0NDE0LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjE2LjQxODE2MTM5MjIxMTkxNCwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI4LjQ3MjcwOTY1NTc2MTcyLCJsZWZ0IjoxMi4zODA4ODAzNTU4MzQ5NjEsIndpZHRoIjozNy4wMTIxODQxNDMwNjY0MDYsImhlaWdodCI6MS4zNjAwMTQ3OTYyNTcwMTl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyOS44NDY1NzI4NzU5NzY1NjIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQwNjQ4NjUxMTIzMDUsImhlaWdodCI6MS4zNjAwMTY3MDM2MDU2NTE5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzEuMjE0MDM2OTQxNTI4MzIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQ0ODU2MjYyMjA3MDMsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzIuNTg3OTAyMDY5MDkxOCwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDYwNTc4OTE4NDU3LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMzLjk2MTc2MTQ3NDYwOTM3NSwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDA0ODQ2MTkxNDA2MjUsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzUuMzM1NjI0Njk0ODI0MjIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzM5NzQ0NTY3ODcxMDk0LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjM2LjcwOTQ4NzkxNTAzOTA2LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjAwMTU5MDcyODc1OTc2NiwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC4wODMzNDczMjA1NTY2NCwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTY1MzA3NjE3MiwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOS40NTcyMTA1NDA3NzE0ODQsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQzMDQ4MDk1NzAzMTI1LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQwLjc0MDU0NzE4MDE3NTc4LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjM0NTQ1NTE2OTY3NzczNCwiaGVpZ2h0IjoxLjQ1MDU0MjgwNzU3OTA0MDV9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Mi4yMDQ5MzMxNjY1MDM5MDYsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMDAzMjUwMTIyMDcwMzEsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDMuNTc4Nzk2Mzg2NzE4NzUsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzM3OTIxMTQyNTc4MTI1LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQ0Ljg2MjEzMzAyNjEyMzA1LCJsZWZ0Ijo5LjA0NzYwMTY5OTgyOTEwMiwid2lkdGgiOjQwLjM0NTMxNDAyNTg3ODkwNiwiaGVpZ2h0IjoxLjQ1MDU0MjgwNzU3OTA0MDV9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ni4zMjY1MTkwMTI0NTExNywibGVmdCI6OS4wNDc2MDE2OTk4MjkxMDIsIndpZHRoIjoyNy45Njc0NDcyODA4ODM3OSwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ny43MDAzODIyMzI2NjYwMTYsImxlZnQiOjEyLjM4MDg3ODQ0ODQ4NjMyOCwid2lkdGgiOjM3LjAzNDc4NjIyNDM2NTIzNCwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0OS4wNzQyNDU0NTI4ODA4NiwibGVmdCI6OS4wNDc2MDE2OTk4MjkxMDIsIndpZHRoIjo0MC4zNDA2NDQ4MzY0MjU3OCwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1MC4zNTc1ODIwOTIyODUxNTYsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzQ0ODUyNDQ3NTA5NzY2LCJoZWlnaHQiOjEuNDUwNTQyODA3NTc5MDQwNX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEzMSJ9fV0%3D.

- Energy reduction: Lower pumping requirements due to precise, low-pressure delivery, addressing agriculture's high energy demands (Daraz et al., 2025)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlVtYXIiLCJmYW1pbHkiOiJEYXJheiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiLFoHRlZmFuIiwiZmFtaWx5IjoiQm9qbmVjIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllvdW5hcyIsImZhbWlseSI6IktoYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMDc3LTA0NzIiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzOTAvYWdyaWN1bHR1cmUxNTA1MDU1NCIsIm9wZW5hbGV4IjoiVzQ0MDgyMTk4MTEiLCJleGEiOiJodHRwczovL3d3dy5tZHBpLmNvbS8yMDc3LTA0NzIvMTUvNS81NTQifSwidGl0bGUiOiJFbmVyZ3ktRWZmaWNpZW50IFNtYXJ0IElycmlnYXRpb24gVGVjaG5vbG9naWVzOiBBIFBhdGh3YXkgdG8gV2F0ZXIgYW5kIEVuZXJneSBTdXN0YWluYWJpbGl0eSBpbiBBZ3JpY3VsdHVyZSIsImFic3RyYWN0IjoiVGhlIGFncmljdWx0dXJhbCBzZWN0b3IgZmFjZXMgY2hhbGxlbmdlcyBzdWNoIGFzIHdhdGVyIHNjYXJjaXR5LCBlbmVyZ3kgaW5lZmZpY2llbmN5LCBhbmQgZGVjbGluaW5nIHByb2R1Y3Rpdml0eSwgcGFydGljdWxhcmx5IGluIGFyaWQgcmVnaW9ucy4gVHJhZGl0aW9uYWwgaXJyaWdhdGlvbiBtZXRob2RzIGNvbnRyaWJ1dGUgdG8gcmVzb3VyY2UgZGVwbGV0aW9uIGFuZCBlbnZpcm9ubWVudGFsIGltcGFjdHMuIFNvbGFyLXBvd2VyZWQgc21hcnQgaXJyaWdhdGlvbiBzeXN0ZW1zIGludGVncmF0ZSBwcmVjaXNpb24gaXJyaWdhdGlvbiB3aXRoIHJlbmV3YWJsZSBlbmVyZ3ksIGltcHJvdmluZyB3YXRlciB1c2UgYW5kIHByb2R1Y3Rpdml0eS4gSW4gUGFraXN0YW4sIHdoZXJlIGFncmljdWx0dXJlIGNvbnRyaWJ1dGVzIDE5JSBvZiBncm9zcyBkb21lc3RpYyBwcm9kdWN0IGFuZCBlbXBsb3lzIDQwJSBvZiB0aGUgd29ya2ZvcmNlLCB0aGVzZSBjaGFsbGVuZ2VzIGFyZSBzZXZlcmUsIGVzcGVjaWFsbHkgaW4gd2F0ZXItc2NhcmNlIGFyZWFzIGxpa2UgdGhlIENob2xpc3RhbiBEZXNlcnQuIFRoaXMgc3R1ZHkgZXhhbWluZXMgdGhlIGltcGFjdCBvZiBzb2xhci1wb3dlcmVkIHNtYXJ0IGlycmlnYXRpb24gb24gYWdyaWN1bHR1cmFsIHByb2R1Y3Rpdml0eSwgd2F0ZXIgY29uc2VydmF0aW9uLCBhbmQgZW5lcmd5IGVmZmljaWVuY3kgaW4gdGhlIENob2xpc3RhbiBEZXNlcnQuIFVzaW5nIGEgcXVhbnRpdGF0aXZlIGNyb3NzLXNlY3Rpb25hbCBkZXNpZ24sIGRhdGEgd2VyZSBjb2xsZWN0ZWQgZnJvbSAzODQgZmFybWVycyB2aWEgc3RydWN0dXJlZCBxdWVzdGlvbm5haXJlcy4gU3RhdGlzdGljYWwgYW5hbHlzZXMsIGluY2x1ZGluZyBtdWx0aXBsZSBsaW5lYXIgcmVncmVzc2lvbiwgcGFpcmVkIHNhbXBsZSB0LXRlc3RzLCBhbmQgU3RydWN0dXJhbCBFcXVhdGlvbiBNb2RlbGluZyAoU0VNKSwgd2VyZSBjb25kdWN0ZWQuIEZpbmRpbmdzIHNob3cgc2lnbmlmaWNhbnQgaW1wcm92ZW1lbnRzIGluIGNyb3AgeWllbGQgKGZyb20gMy4wIHRvIDQuOCB0b25zL2hlY3RhcmUpIGFuZCByZWR1Y3Rpb25zIGluIHdhdGVyIGFuZCBlbmVyZ3kgY29uc3VtcHRpb24uIFJlZ3Jlc3Npb24gYW5hbHlzaXMgaGlnaGxpZ2h0ZWQgc3Ryb25nIHBvc2l0aXZlIGVmZmVjdHMgb24geWllbGQgYW5kIGVmZmljaWVuY3ksIHdoaWxlIFNFTSBjb25maXJtZWQgcmVkdWNlZCBlbnZpcm9ubWVudGFsIGltcGFjdCBhbmQgb3BlcmF0aW9uYWwgY29zdHMuIFRoZSBzdHVkeSBjb25jbHVkZXMgdGhhdCBzb2xhci1wb3dlcmVkIGlycmlnYXRpb24gZW5oYW5jZXMgcHJvZHVjdGl2aXR5LCBjb25zZXJ2ZXMgcmVzb3VyY2VzLCBhbmQgcHJvbW90ZXMgc3VzdGFpbmFiaWxpdHkuIFBvbGljeW1ha2VycyBzaG91bGQgcHJvdmlkZSBmaW5hbmNpYWwgaW5jZW50aXZlcywgaW52ZXN0IGluIHJlbmV3YWJsZSBpbmZyYXN0cnVjdHVyZSwgYW5kIGltcGxlbWVudCB0cmFpbmluZyBwcm9ncmFtcyB0byBzdXBwb3J0IGFkb3B0aW9uLiBDb2xsYWJvcmF0aXZlIGVmZm9ydHMgYXJlIGVzc2VudGlhbCBmb3Igc3VzdGFpbmFibGUgYWdyaWN1bHR1cmUgaW4gYXJpZCByZWdpb25zLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJpc3N1ZWREYXRlIjoiMjAyNS0wMy0wNSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkFncmljdWx0dXJlIiwiaXNzdWUiOjUsInZvbHVtZSI6MTUsImltcGFjdEZhY3RvciI6IjUuMjYifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjo1NTQsImxhc3RQYWdlIjo1NTR9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiTXVsdGlkaXNjaXBsaW5hcnkgRGlnaXRhbCBQdWJsaXNoaW5nIEluc3RpdHV0ZSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NDgsImNpdGVkQnlDb3VudCI6Mn19LCJxdW90ZSI6IjEuIEludHJvZHVjdGlvbiBUaGUgYWdyaWN1bHR1cmFsIHNlY3RvciwgYSBjb3JuZXJzdG9uZSBvZiBnbG9iYWwgZm9vZCBzZWN1cml0eSBhbmQgZWNvbm9taWMgZGV2ZWxvcG1lbnQsIGZhY2VzIG1vdW50aW5nIGNoYWxsZW5nZXMgaW4gdGhlIDIxc3QgY2VudHVyeS4gQ2xpbWF0ZSBjaGFuZ2UsIHJhcGlkIHBvcHVsYXRpb24gZ3Jvd3RoLCBhbmQgZHdpbmRsaW5nIG5hdHVyYWwgcmVzb3VyY2VzIGhhdmUgaW50ZW5zaWZpZWQgdGhlIG5lZWQgZm9yIHN1c3RhaW5hYmxlIGFncmljdWx0dXJhbCBwcmFjdGljZXMuIEFtb25nIHRoZXNlIGNoYWxsZW5nZXMsIHdhdGVyIHNjYXJjaXR5IGFuZCBlbmVyZ3kgaW5lZmZpY2llbmN5IHN0YW5kIG91dCBhcyBjcml0aWNhbCBiYXJyaWVycyB0byBwcm9kdWN0aXZpdHkgYW5kIHN1c3RhaW5hYmlsaXR5LiBHbG9iYWxseSwgYWdyaWN1bHR1cmUgYWNjb3VudHMgZm9yIGFwcHJveGltYXRlbHkgNzAlIG9mIGZyZXNod2F0ZXIgd2l0aGRyYXdhbHMgYW5kIGlzIG9uZSBvZiB0aGUgbGFyZ2VzdCBjb25zdW1lcnMgb2YgZW5lcmd5IGluIG1hbnkgcmVnaW9ucyBbIDEsIDJdLiBUcmFkaXRpb25hbCBpcnJpZ2F0aW9uIHByYWN0aWNlcywgb2Z0ZW4gaW5lZmZpY2llbnQgYW5kIHJlc291cmNlLWludGVuc2l2ZSwgZXhhY2VyYmF0ZSB0aGVzZSBpc3N1ZXMsIGxlYWRpbmcgdG8gd2F0ZXIgd2FzdGFnZSwgZW5lcmd5IG92ZXJ1c2UsIGFuZCByZWR1Y2VkIGNyb3AgeWllbGRzLiJ9XQ%3D%3D.

- Crop yield improvements: Uniform wetting boosts productivity and quality, mitigating issues like salinization and nutrient leaching (Jumintono et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikp1bWludG9ubyIsImZhbWlseSI6Ikp1bWludG9ubyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJOdXJuYXdhdHkiLCJmYW1pbHkiOiJOdXJuYXdhdHkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTnVncm9obyBUcmlzbnUiLCJmYW1pbHkiOiJCcmF0YSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJEaW5pIiwiZmFtaWx5IjoiUmFobWlhdGkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSGFkaSIsImZhbWlseSI6IlN1c2lsbyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBLiBBLiIsImZhbWlseSI6Ikh1c3NlbiJ9fV0sImlkcyI6eyJpc3NuIjpbIjE0MjktNzQyNiIsIjIwODMtNDUzNSJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMjQ0MjUvandsZC4yMDI0LjE1MTc5OCIsIm9wZW5hbGV4IjoiVzQ0MDU0NzcxMDAifSwidGl0bGUiOiJDb21wYXJhdGl2ZSBhbmFseXNpcyBvZiBwcmVjaXNpb24sIGRlZmljaXQsIGFuZCBkcmlwIGlycmlnYXRpb24gZm9yIHdhdGVyIGVmZmljaWVuY3kgaW4gSXJhcSIsImFic3RyYWN0IjoiU3VzdGFpbmFibGUgaXJyaWdhdGlvbiBwcmFjdGljZXMgYXJlIGNyaXRpY2FsIGZvciBhZGRyZXNzaW5nIHRoZSBjaGFsbGVuZ2VzIG9mIHdhdGVyIHNjYXJjaXR5IGFuZCBmb29kIGluc2VjdXJpdHkgaW4gdGhlIGZhY2Ugb2YgY2xpbWF0ZSBjaGFuZ2UgYW5kIGdyb3dpbmcgZ2xvYmFsIGZvb2QgZGVtYW5kLiBUaGlzIHN0dWR5IGludmVzdGlnYXRlZCB0aGUgcG90ZW50aWFsIG9mIHByZWNpc2lvbiBpcnJpZ2F0aW9uIChQSSksIGRlZmljaXQgaXJyaWdhdGlvbiAoREkpLCBhbmQgZHJpcCBpcnJpZ2F0aW9uIChEUkkpIHRlY2huaXF1ZXMgdG8gZW5oYW5jZSBjcm9wIHByb2R1Y3Rpdml0eSBhbmQgd2F0ZXIgdXNlIGVmZmljaWVuY3kgKFdVRSkgY29tcGFyZWQgdG8gY29udmVudGlvbmFsIGZsb29kIGlycmlnYXRpb24gKENGSSkgaW4gdGhyZWUgZGlzdGluY3QgYWdyb2Vjb2xvZ2ljYWwgem9uZXMgb2YgSXJhcS4gRmllbGQgZXhwZXJpbWVudHMgd2VyZSBjb25kdWN0ZWQgdXNpbmcgYSByYW5kb21pc2VkIGNvbXBsZXRlIGJsb2NrIGRlc2lnbiB3aXRoIHdoZWF0LCBtYWl6ZSwgYW5kIHJpY2UgY3JvcHMuIFRoZSByZXN1bHRzIGRlbW9uc3RyYXRlZCB0aGF0IFBJIHNpZ25pZmljYW50bHkgaW5jcmVhc2VkIGNyb3AgZ3Jvd3RoIHBhcmFtZXRlcnMsIGdyYWluIHlpZWxkLCBhbmQgV1VFIGFjcm9zcyBhbGwgem9uZXMsIHdpdGggeWllbGQgaW1wcm92ZW1lbnRzIG9mIDMz4oCTMzglIGFuZCBXVUUgaW5jcmVhc2VzIG9mIDQ24oCTNTElIGluIGNvbnRyYXN0IHdpdGggQ0ZJLiBUaGUgREkgYW5kIERSSSB0cmVhdG1lbnRzIGFsc28gb3V0cGVyZm9ybWVkIENGSSwgYWxiZWl0IHRvIGEgbGVzc2VyIGV4dGVudC4gUmVtb3RlIHNlbnNpbmctZGVyaXZlZCB2ZWdldGF0aW9uIGluZGljZXMgc3Ryb25nbHkgY29ycmVsYXRlZCB3aXRoIGNyb3AgZ3Jvd3RoIHBhcmFtZXRlcnMgYW5kIHlpZWxkLCB3aGlsZSBoeWRyb2xvZ2ljYWwgbW9kZWxsaW5nIHJldmVhbGVkIHJlZHVjZWQgZXZhcG90cmFuc3BpcmF0aW9uIGFuZCBzdXJmYWNlIHJ1bm9mZiB1bmRlciB0aGUgUEkgdHJlYXRtZW50LiBUaGUgc3VzdGFpbmFibGUgaXJyaWdhdGlvbiBwcmFjdGljZXMgcmVzdWx0ZWQgaW4gc3Vic3RhbnRpYWwgd2F0ZXIgc2F2aW5ncyBvZiAyMOKAkzMwJSBjb21wYXJlZCB0byBDRkkuIFRoZXNlIGZpbmRpbmdzIGhpZ2hsaWdodCB0aGUgaW1wb3J0YW5jZSBvZiBhZG9wdGluZyBlZmZpY2llbnQgaXJyaWdhdGlvbiB0ZWNobmlxdWVzLCBhbG9uZyB3aXRoIGEgaG9saXN0aWMgYXBwcm9hY2ggZW5jb21wYXNzaW5nIHRlY2hub2xvZ2ljYWwgaW5ub3ZhdGlvbnMsIGNhcGFjaXR5IGJ1aWxkaW5nLCBhbmQgc3Rha2Vob2xkZXIgZW5nYWdlbWVudCwgdG8gcHJvbW90ZSB3YXRlci0gZWZmaWNpZW50IGFncmljdWx0dXJlIGFuZCBlbnN1cmUgZm9vZCBzZWN1cml0eSBpbiB3YXRlci1zY2FyY2UgcmVnaW9ucy4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjQ0MjUvandsZC4yMDI0LjE1MTc5OCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjI0NDI1L2p3bGQuMjAyNC4xNTE3OTgiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMi0xNCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkpvdXJuYWwgb2YgV2F0ZXIgYW5kIExhbmQgRGV2ZWxvcG1lbnQiLCJpbXBhY3RGYWN0b3IiOiIxLjQ2In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTMwLCJsYXN0UGFnZSI6MTM2fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkRlIEdydXl0ZXIgT3BlbiJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MzIsImNpdGVkQnlDb3VudCI6MX19LCJxdW90ZSI6IkluIHRoaXMgY29udGV4dCwgdGhlIGFkb3B0aW9uIG9mICBzdXN0YWluYWJsZSBpcnJpZ2F0aW9uIHByYWN0aWNlcyBoYXMgZW1lcmdlZCBhcyBhIGNyaXRpY2FsIHN0cmF0ZWd5ICBmb3IgZW5oYW5jaW5nIGZvb2Qgc2VjdXJpdHkgd2hpbGUgcHJvbW90aW5nIHdhdGVyIGNvbnNlcnZhdGlvbiAgaW4gYWdyaWN1bHR1cmFsIGxhbmRzY2FwZXMuICBJcnJpZ2F0aW9uIHBsYXlzIGEgdml0YWwgcm9sZSBpbiBnbG9iYWwgZm9vZCBwcm9kdWN0aW9uLCB3aXRoICBpcnJpZ2F0ZWQgYWdyaWN1bHR1cmUgYWNjb3VudGluZyBmb3IgYXBwcm94aW1hdGVseSA0MCUgb2YgdGhlICBmb29kIHN1cHBseSAoV3JhY2hpZW4gZGUsIFNjaHVsdHogYW5kIEdvbGksIDIwMjE7IFNjaG1pdHQsIFJvc2EgIGFuZCBEYWlseSwgMjAyMikuIEhvd2V2ZXIsIHRyYWRpdGlvbmFsIGlycmlnYXRpb24gbWV0aG9kcywgc3VjaCBhcyAgZmxvb2QgYW5kIGZ1cnJvdyBpcnJpZ2F0aW9uLCBhcmUgb2Z0ZW4gaW5lZmZpY2llbnQsIGxlYWRpbmcgdG8gIHN1YnN0YW50aWFsIHdhdGVyIGxvc3NlcyB0aHJvdWdoIGV2YXBvcmF0aW9uLCBydW5vZmYsIGFuZCBkZWVwICBwZXJjb2xhdGlvbiAoQW1ib21zYSwgMjAyMCkuIE1vcmVvdmVyLCB0aGUgZXhjZXNzaXZlIGFwcGxpY2EtdGlvbiBvZiB3YXRlciBjYW4gcmVzdWx0IGluIHNvaWwgc2FsaW5pc2F0aW9uLCBudXRyaWVudCBsZWFjaGluZywgYW5kICBncm91bmR3YXRlciBkZXBsZXRpb24sIGZ1cnRoZXIgY29tcHJvbWlzaW5nIHRoZSBsb25nLXRlcm0gIHN1c3RhaW5hYmlsaXR5IG9mIGFncmljdWx0dXJhbCBzeXN0ZW1zIChZaW4gZXQgYWwuLCAyMDIyKS4gVG8gYWRkcmVzcyAgdGhlc2UgY2hhbGxlbmdlcywgcmVzZWFyY2hlcnMgYW5kIHByYWN0aXRpb25lcnMgaGF2ZSBiZWVuIGV4cGxvci1pbmcgaW5ub3ZhdGl2ZSBpcnJpZ2F0aW9uIHRlY2huaXF1ZXMgYW5kIHdhdGVyIG1hbmFnZW1lbnQgIHN0cmF0ZWdpZXMgdGhhdCBjYW4gb3B0aW1pc2Ugd2F0ZXIgdXNlIGVmZmljaWVuY3kgKFdVRSkgd2hpbGUgIG1haW50YWluaW5nIG9yIGVuaGFuY2luZyBjcm9wIHByb2R1Y3Rpdml0eS4gIFByZWNpc2lvbiBpcnJpZ2F0aW9uIChQSSkgb2ZmZXJzIGEgcHJvbWlzaW5nIHNvbHV0aW9uLCAgaW52b2x2aW5nIHRoZSB0YXJnZXRlZCBhcHBsaWNhdGlvbiBvZiB3YXRlciBzcGVjaWZpY2FsbHkgdG8gdGhlICBuZWVkcyBvZiBpbmRpdmlkdWFsIHBsYW50cyBvciBjcm9wIHpvbmVzIChaaW5rZXJuYWdlbCBldCBhbC4sIDIwMjApLiAgIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyMi44ODY3MzU5MTYxMzc2OTUsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzQ2MjIxOTIzODI4MTI1LCJoZWlnaHQiOjEuNDUwNTQxMDE5NDM5Njk3M319LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI0LjM1MTEyMzgwOTgxNDQ1MywibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTI3MTYwNjQ0NSwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI1LjcyNDk4NzAzMDAyOTI5NywibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTY1MzA3NjE3MiwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI3LjA5ODg1MDI1MDI0NDE0LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjE2LjQxODE2MTM5MjIxMTkxNCwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI4LjQ3MjcwOTY1NTc2MTcyLCJsZWZ0IjoxMi4zODA4ODAzNTU4MzQ5NjEsIndpZHRoIjozNy4wMTIxODQxNDMwNjY0MDYsImhlaWdodCI6MS4zNjAwMTQ3OTYyNTcwMTl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyOS44NDY1NzI4NzU5NzY1NjIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQwNjQ4NjUxMTIzMDUsImhlaWdodCI6MS4zNjAwMTY3MDM2MDU2NTE5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzEuMjE0MDM2OTQxNTI4MzIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQ0ODU2MjYyMjA3MDMsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzIuNTg3OTAyMDY5MDkxOCwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDYwNTc4OTE4NDU3LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMzLjk2MTc2MTQ3NDYwOTM3NSwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDA0ODQ2MTkxNDA2MjUsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzUuMzM1NjI0Njk0ODI0MjIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzM5NzQ0NTY3ODcxMDk0LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjM2LjcwOTQ4NzkxNTAzOTA2LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjAwMTU5MDcyODc1OTc2NiwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC4wODMzNDczMjA1NTY2NCwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTY1MzA3NjE3MiwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOS40NTcyMTA1NDA3NzE0ODQsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQzMDQ4MDk1NzAzMTI1LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQwLjc0MDU0NzE4MDE3NTc4LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjM0NTQ1NTE2OTY3NzczNCwiaGVpZ2h0IjoxLjQ1MDU0MjgwNzU3OTA0MDV9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Mi4yMDQ5MzMxNjY1MDM5MDYsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMDAzMjUwMTIyMDcwMzEsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDMuNTc4Nzk2Mzg2NzE4NzUsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzM3OTIxMTQyNTc4MTI1LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQ0Ljg2MjEzMzAyNjEyMzA1LCJsZWZ0Ijo5LjA0NzYwMTY5OTgyOTEwMiwid2lkdGgiOjQwLjM0NTMxNDAyNTg3ODkwNiwiaGVpZ2h0IjoxLjQ1MDU0MjgwNzU3OTA0MDV9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ni4zMjY1MTkwMTI0NTExNywibGVmdCI6OS4wNDc2MDE2OTk4MjkxMDIsIndpZHRoIjoyNy45Njc0NDcyODA4ODM3OSwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ny43MDAzODIyMzI2NjYwMTYsImxlZnQiOjEyLjM4MDg3ODQ0ODQ4NjMyOCwid2lkdGgiOjM3LjAzNDc4NjIyNDM2NTIzNCwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0OS4wNzQyNDU0NTI4ODA4NiwibGVmdCI6OS4wNDc2MDE2OTk4MjkxMDIsIndpZHRoIjo0MC4zNDA2NDQ4MzY0MjU3OCwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1MC4zNTc1ODIwOTIyODUxNTYsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzQ0ODUyNDQ3NTA5NzY2LCJoZWlnaHQiOjEuNDUwNTQyODA3NTc5MDQwNX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEzMSJ9fV0%3D.

- Environmental benefits: Minimize soil erosion, weed growth, and disease by keeping foliage dry.

Applications and Implementation

The technique applicable for in row crops of horticulture and plantation crops, greenhouses, and arid regions, these systems integrate with automation for site-specific management. This technique can also be used in field crops by altering the planting geometry (Paired row technique) In water-scarce areas like Iraq, they enhance WUE under deficit strategies (Jumintono et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikp1bWludG9ubyIsImZhbWlseSI6Ikp1bWludG9ubyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJOdXJuYXdhdHkiLCJmYW1pbHkiOiJOdXJuYXdhdHkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTnVncm9obyBUcmlzbnUiLCJmYW1pbHkiOiJCcmF0YSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJEaW5pIiwiZmFtaWx5IjoiUmFobWlhdGkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSGFkaSIsImZhbWlseSI6IlN1c2lsbyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBLiBBLiIsImZhbWlseSI6Ikh1c3NlbiJ9fV0sImlkcyI6eyJpc3NuIjpbIjE0MjktNzQyNiIsIjIwODMtNDUzNSJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMjQ0MjUvandsZC4yMDI0LjE1MTc5OCIsIm9wZW5hbGV4IjoiVzQ0MDU0NzcxMDAifSwidGl0bGUiOiJDb21wYXJhdGl2ZSBhbmFseXNpcyBvZiBwcmVjaXNpb24sIGRlZmljaXQsIGFuZCBkcmlwIGlycmlnYXRpb24gZm9yIHdhdGVyIGVmZmljaWVuY3kgaW4gSXJhcSIsImFic3RyYWN0IjoiU3VzdGFpbmFibGUgaXJyaWdhdGlvbiBwcmFjdGljZXMgYXJlIGNyaXRpY2FsIGZvciBhZGRyZXNzaW5nIHRoZSBjaGFsbGVuZ2VzIG9mIHdhdGVyIHNjYXJjaXR5IGFuZCBmb29kIGluc2VjdXJpdHkgaW4gdGhlIGZhY2Ugb2YgY2xpbWF0ZSBjaGFuZ2UgYW5kIGdyb3dpbmcgZ2xvYmFsIGZvb2QgZGVtYW5kLiBUaGlzIHN0dWR5IGludmVzdGlnYXRlZCB0aGUgcG90ZW50aWFsIG9mIHByZWNpc2lvbiBpcnJpZ2F0aW9uIChQSSksIGRlZmljaXQgaXJyaWdhdGlvbiAoREkpLCBhbmQgZHJpcCBpcnJpZ2F0aW9uIChEUkkpIHRlY2huaXF1ZXMgdG8gZW5oYW5jZSBjcm9wIHByb2R1Y3Rpdml0eSBhbmQgd2F0ZXIgdXNlIGVmZmljaWVuY3kgKFdVRSkgY29tcGFyZWQgdG8gY29udmVudGlvbmFsIGZsb29kIGlycmlnYXRpb24gKENGSSkgaW4gdGhyZWUgZGlzdGluY3QgYWdyb2Vjb2xvZ2ljYWwgem9uZXMgb2YgSXJhcS4gRmllbGQgZXhwZXJpbWVudHMgd2VyZSBjb25kdWN0ZWQgdXNpbmcgYSByYW5kb21pc2VkIGNvbXBsZXRlIGJsb2NrIGRlc2lnbiB3aXRoIHdoZWF0LCBtYWl6ZSwgYW5kIHJpY2UgY3JvcHMuIFRoZSByZXN1bHRzIGRlbW9uc3RyYXRlZCB0aGF0IFBJIHNpZ25pZmljYW50bHkgaW5jcmVhc2VkIGNyb3AgZ3Jvd3RoIHBhcmFtZXRlcnMsIGdyYWluIHlpZWxkLCBhbmQgV1VFIGFjcm9zcyBhbGwgem9uZXMsIHdpdGggeWllbGQgaW1wcm92ZW1lbnRzIG9mIDMz4oCTMzglIGFuZCBXVUUgaW5jcmVhc2VzIG9mIDQ24oCTNTElIGluIGNvbnRyYXN0IHdpdGggQ0ZJLiBUaGUgREkgYW5kIERSSSB0cmVhdG1lbnRzIGFsc28gb3V0cGVyZm9ybWVkIENGSSwgYWxiZWl0IHRvIGEgbGVzc2VyIGV4dGVudC4gUmVtb3RlIHNlbnNpbmctZGVyaXZlZCB2ZWdldGF0aW9uIGluZGljZXMgc3Ryb25nbHkgY29ycmVsYXRlZCB3aXRoIGNyb3AgZ3Jvd3RoIHBhcmFtZXRlcnMgYW5kIHlpZWxkLCB3aGlsZSBoeWRyb2xvZ2ljYWwgbW9kZWxsaW5nIHJldmVhbGVkIHJlZHVjZWQgZXZhcG90cmFuc3BpcmF0aW9uIGFuZCBzdXJmYWNlIHJ1bm9mZiB1bmRlciB0aGUgUEkgdHJlYXRtZW50LiBUaGUgc3VzdGFpbmFibGUgaXJyaWdhdGlvbiBwcmFjdGljZXMgcmVzdWx0ZWQgaW4gc3Vic3RhbnRpYWwgd2F0ZXIgc2F2aW5ncyBvZiAyMOKAkzMwJSBjb21wYXJlZCB0byBDRkkuIFRoZXNlIGZpbmRpbmdzIGhpZ2hsaWdodCB0aGUgaW1wb3J0YW5jZSBvZiBhZG9wdGluZyBlZmZpY2llbnQgaXJyaWdhdGlvbiB0ZWNobmlxdWVzLCBhbG9uZyB3aXRoIGEgaG9saXN0aWMgYXBwcm9hY2ggZW5jb21wYXNzaW5nIHRlY2hub2xvZ2ljYWwgaW5ub3ZhdGlvbnMsIGNhcGFjaXR5IGJ1aWxkaW5nLCBhbmQgc3Rha2Vob2xkZXIgZW5nYWdlbWVudCwgdG8gcHJvbW90ZSB3YXRlci0gZWZmaWNpZW50IGFncmljdWx0dXJlIGFuZCBlbnN1cmUgZm9vZCBzZWN1cml0eSBpbiB3YXRlci1zY2FyY2UgcmVnaW9ucy4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjQ0MjUvandsZC4yMDI0LjE1MTc5OCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjI0NDI1L2p3bGQuMjAyNC4xNTE3OTgiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMi0xNCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkpvdXJuYWwgb2YgV2F0ZXIgYW5kIExhbmQgRGV2ZWxvcG1lbnQiLCJpbXBhY3RGYWN0b3IiOiIxLjQ2In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTMwLCJsYXN0UGFnZSI6MTM2fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkRlIEdydXl0ZXIgT3BlbiJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MzIsImNpdGVkQnlDb3VudCI6MX19LCJxdW90ZSI6IkluIHRoaXMgY29udGV4dCwgdGhlIGFkb3B0aW9uIG9mICBzdXN0YWluYWJsZSBpcnJpZ2F0aW9uIHByYWN0aWNlcyBoYXMgZW1lcmdlZCBhcyBhIGNyaXRpY2FsIHN0cmF0ZWd5ICBmb3IgZW5oYW5jaW5nIGZvb2Qgc2VjdXJpdHkgd2hpbGUgcHJvbW90aW5nIHdhdGVyIGNvbnNlcnZhdGlvbiAgaW4gYWdyaWN1bHR1cmFsIGxhbmRzY2FwZXMuICBJcnJpZ2F0aW9uIHBsYXlzIGEgdml0YWwgcm9sZSBpbiBnbG9iYWwgZm9vZCBwcm9kdWN0aW9uLCB3aXRoICBpcnJpZ2F0ZWQgYWdyaWN1bHR1cmUgYWNjb3VudGluZyBmb3IgYXBwcm94aW1hdGVseSA0MCUgb2YgdGhlICBmb29kIHN1cHBseSAoV3JhY2hpZW4gZGUsIFNjaHVsdHogYW5kIEdvbGksIDIwMjE7IFNjaG1pdHQsIFJvc2EgIGFuZCBEYWlseSwgMjAyMikuIEhvd2V2ZXIsIHRyYWRpdGlvbmFsIGlycmlnYXRpb24gbWV0aG9kcywgc3VjaCBhcyAgZmxvb2QgYW5kIGZ1cnJvdyBpcnJpZ2F0aW9uLCBhcmUgb2Z0ZW4gaW5lZmZpY2llbnQsIGxlYWRpbmcgdG8gIHN1YnN0YW50aWFsIHdhdGVyIGxvc3NlcyB0aHJvdWdoIGV2YXBvcmF0aW9uLCBydW5vZmYsIGFuZCBkZWVwICBwZXJjb2xhdGlvbiAoQW1ib21zYSwgMjAyMCkuIE1vcmVvdmVyLCB0aGUgZXhjZXNzaXZlIGFwcGxpY2EtdGlvbiBvZiB3YXRlciBjYW4gcmVzdWx0IGluIHNvaWwgc2FsaW5pc2F0aW9uLCBudXRyaWVudCBsZWFjaGluZywgYW5kICBncm91bmR3YXRlciBkZXBsZXRpb24sIGZ1cnRoZXIgY29tcHJvbWlzaW5nIHRoZSBsb25nLXRlcm0gIHN1c3RhaW5hYmlsaXR5IG9mIGFncmljdWx0dXJhbCBzeXN0ZW1zIChZaW4gZXQgYWwuLCAyMDIyKS4gVG8gYWRkcmVzcyAgdGhlc2UgY2hhbGxlbmdlcywgcmVzZWFyY2hlcnMgYW5kIHByYWN0aXRpb25lcnMgaGF2ZSBiZWVuIGV4cGxvci1pbmcgaW5ub3ZhdGl2ZSBpcnJpZ2F0aW9uIHRlY2huaXF1ZXMgYW5kIHdhdGVyIG1hbmFnZW1lbnQgIHN0cmF0ZWdpZXMgdGhhdCBjYW4gb3B0aW1pc2Ugd2F0ZXIgdXNlIGVmZmljaWVuY3kgKFdVRSkgd2hpbGUgIG1haW50YWluaW5nIG9yIGVuaGFuY2luZyBjcm9wIHByb2R1Y3Rpdml0eS4gIFByZWNpc2lvbiBpcnJpZ2F0aW9uIChQSSkgb2ZmZXJzIGEgcHJvbWlzaW5nIHNvbHV0aW9uLCAgaW52b2x2aW5nIHRoZSB0YXJnZXRlZCBhcHBsaWNhdGlvbiBvZiB3YXRlciBzcGVjaWZpY2FsbHkgdG8gdGhlICBuZWVkcyBvZiBpbmRpdmlkdWFsIHBsYW50cyBvciBjcm9wIHpvbmVzIChaaW5rZXJuYWdlbCBldCBhbC4sIDIwMjApLiAgIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyMi44ODY3MzU5MTYxMzc2OTUsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzQ2MjIxOTIzODI4MTI1LCJoZWlnaHQiOjEuNDUwNTQxMDE5NDM5Njk3M319LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI0LjM1MTEyMzgwOTgxNDQ1MywibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTI3MTYwNjQ0NSwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI1LjcyNDk4NzAzMDAyOTI5NywibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTY1MzA3NjE3MiwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI3LjA5ODg1MDI1MDI0NDE0LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjE2LjQxODE2MTM5MjIxMTkxNCwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI4LjQ3MjcwOTY1NTc2MTcyLCJsZWZ0IjoxMi4zODA4ODAzNTU4MzQ5NjEsIndpZHRoIjozNy4wMTIxODQxNDMwNjY0MDYsImhlaWdodCI6MS4zNjAwMTQ3OTYyNTcwMTl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyOS44NDY1NzI4NzU5NzY1NjIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQwNjQ4NjUxMTIzMDUsImhlaWdodCI6MS4zNjAwMTY3MDM2MDU2NTE5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzEuMjE0MDM2OTQxNTI4MzIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQ0ODU2MjYyMjA3MDMsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzIuNTg3OTAyMDY5MDkxOCwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDYwNTc4OTE4NDU3LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMzLjk2MTc2MTQ3NDYwOTM3NSwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDA0ODQ2MTkxNDA2MjUsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzUuMzM1NjI0Njk0ODI0MjIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzM5NzQ0NTY3ODcxMDk0LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjM2LjcwOTQ4NzkxNTAzOTA2LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjAwMTU5MDcyODc1OTc2NiwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC4wODMzNDczMjA1NTY2NCwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTY1MzA3NjE3MiwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOS40NTcyMTA1NDA3NzE0ODQsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQzMDQ4MDk1NzAzMTI1LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQwLjc0MDU0NzE4MDE3NTc4LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjM0NTQ1NTE2OTY3NzczNCwiaGVpZ2h0IjoxLjQ1MDU0MjgwNzU3OTA0MDV9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Mi4yMDQ5MzMxNjY1MDM5MDYsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMDAzMjUwMTIyMDcwMzEsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDMuNTc4Nzk2Mzg2NzE4NzUsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzM3OTIxMTQyNTc4MTI1LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQ0Ljg2MjEzMzAyNjEyMzA1LCJsZWZ0Ijo5LjA0NzYwMTY5OTgyOTEwMiwid2lkdGgiOjQwLjM0NTMxNDAyNTg3ODkwNiwiaGVpZ2h0IjoxLjQ1MDU0MjgwNzU3OTA0MDV9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ni4zMjY1MTkwMTI0NTExNywibGVmdCI6OS4wNDc2MDE2OTk4MjkxMDIsIndpZHRoIjoyNy45Njc0NDcyODA4ODM3OSwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ny43MDAzODIyMzI2NjYwMTYsImxlZnQiOjEyLjM4MDg3ODQ0ODQ4NjMyOCwid2lkdGgiOjM3LjAzNDc4NjIyNDM2NTIzNCwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0OS4wNzQyNDU0NTI4ODA4NiwibGVmdCI6OS4wNDc2MDE2OTk4MjkxMDIsIndpZHRoIjo0MC4zNDA2NDQ4MzY0MjU3OCwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1MC4zNTc1ODIwOTIyODUxNTYsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzQ0ODUyNDQ3NTA5NzY2LCJoZWlnaHQiOjEuNDUwNTQyODA3NTc5MDQwNX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEzMSJ9fV0%3D.

Figure 5: The IoT-Based Smart Irrigation Architecture

### Sprinkler and Pivot Irrigation Systems

Sprinkler and pivot irrigation systems simulate rainfall by distributing water through overhead sprinklers or rotating pivots, achieving efficiencies of 70-85% by reducing surface runoff and deep percolation compared to traditional flood methods (Daraz et al., 2025)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlVtYXIiLCJmYW1pbHkiOiJEYXJheiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiLFoHRlZmFuIiwiZmFtaWx5IjoiQm9qbmVjIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllvdW5hcyIsImZhbWlseSI6IktoYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMDc3LTA0NzIiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzOTAvYWdyaWN1bHR1cmUxNTA1MDU1NCIsIm9wZW5hbGV4IjoiVzQ0MDgyMTk4MTEiLCJleGEiOiJodHRwczovL3d3dy5tZHBpLmNvbS8yMDc3LTA0NzIvMTUvNS81NTQifSwidGl0bGUiOiJFbmVyZ3ktRWZmaWNpZW50IFNtYXJ0IElycmlnYXRpb24gVGVjaG5vbG9naWVzOiBBIFBhdGh3YXkgdG8gV2F0ZXIgYW5kIEVuZXJneSBTdXN0YWluYWJpbGl0eSBpbiBBZ3JpY3VsdHVyZSIsImFic3RyYWN0IjoiVGhlIGFncmljdWx0dXJhbCBzZWN0b3IgZmFjZXMgY2hhbGxlbmdlcyBzdWNoIGFzIHdhdGVyIHNjYXJjaXR5LCBlbmVyZ3kgaW5lZmZpY2llbmN5LCBhbmQgZGVjbGluaW5nIHByb2R1Y3Rpdml0eSwgcGFydGljdWxhcmx5IGluIGFyaWQgcmVnaW9ucy4gVHJhZGl0aW9uYWwgaXJyaWdhdGlvbiBtZXRob2RzIGNvbnRyaWJ1dGUgdG8gcmVzb3VyY2UgZGVwbGV0aW9uIGFuZCBlbnZpcm9ubWVudGFsIGltcGFjdHMuIFNvbGFyLXBvd2VyZWQgc21hcnQgaXJyaWdhdGlvbiBzeXN0ZW1zIGludGVncmF0ZSBwcmVjaXNpb24gaXJyaWdhdGlvbiB3aXRoIHJlbmV3YWJsZSBlbmVyZ3ksIGltcHJvdmluZyB3YXRlciB1c2UgYW5kIHByb2R1Y3Rpdml0eS4gSW4gUGFraXN0YW4sIHdoZXJlIGFncmljdWx0dXJlIGNvbnRyaWJ1dGVzIDE5JSBvZiBncm9zcyBkb21lc3RpYyBwcm9kdWN0IGFuZCBlbXBsb3lzIDQwJSBvZiB0aGUgd29ya2ZvcmNlLCB0aGVzZSBjaGFsbGVuZ2VzIGFyZSBzZXZlcmUsIGVzcGVjaWFsbHkgaW4gd2F0ZXItc2NhcmNlIGFyZWFzIGxpa2UgdGhlIENob2xpc3RhbiBEZXNlcnQuIFRoaXMgc3R1ZHkgZXhhbWluZXMgdGhlIGltcGFjdCBvZiBzb2xhci1wb3dlcmVkIHNtYXJ0IGlycmlnYXRpb24gb24gYWdyaWN1bHR1cmFsIHByb2R1Y3Rpdml0eSwgd2F0ZXIgY29uc2VydmF0aW9uLCBhbmQgZW5lcmd5IGVmZmljaWVuY3kgaW4gdGhlIENob2xpc3RhbiBEZXNlcnQuIFVzaW5nIGEgcXVhbnRpdGF0aXZlIGNyb3NzLXNlY3Rpb25hbCBkZXNpZ24sIGRhdGEgd2VyZSBjb2xsZWN0ZWQgZnJvbSAzODQgZmFybWVycyB2aWEgc3RydWN0dXJlZCBxdWVzdGlvbm5haXJlcy4gU3RhdGlzdGljYWwgYW5hbHlzZXMsIGluY2x1ZGluZyBtdWx0aXBsZSBsaW5lYXIgcmVncmVzc2lvbiwgcGFpcmVkIHNhbXBsZSB0LXRlc3RzLCBhbmQgU3RydWN0dXJhbCBFcXVhdGlvbiBNb2RlbGluZyAoU0VNKSwgd2VyZSBjb25kdWN0ZWQuIEZpbmRpbmdzIHNob3cgc2lnbmlmaWNhbnQgaW1wcm92ZW1lbnRzIGluIGNyb3AgeWllbGQgKGZyb20gMy4wIHRvIDQuOCB0b25zL2hlY3RhcmUpIGFuZCByZWR1Y3Rpb25zIGluIHdhdGVyIGFuZCBlbmVyZ3kgY29uc3VtcHRpb24uIFJlZ3Jlc3Npb24gYW5hbHlzaXMgaGlnaGxpZ2h0ZWQgc3Ryb25nIHBvc2l0aXZlIGVmZmVjdHMgb24geWllbGQgYW5kIGVmZmljaWVuY3ksIHdoaWxlIFNFTSBjb25maXJtZWQgcmVkdWNlZCBlbnZpcm9ubWVudGFsIGltcGFjdCBhbmQgb3BlcmF0aW9uYWwgY29zdHMuIFRoZSBzdHVkeSBjb25jbHVkZXMgdGhhdCBzb2xhci1wb3dlcmVkIGlycmlnYXRpb24gZW5oYW5jZXMgcHJvZHVjdGl2aXR5LCBjb25zZXJ2ZXMgcmVzb3VyY2VzLCBhbmQgcHJvbW90ZXMgc3VzdGFpbmFiaWxpdHkuIFBvbGljeW1ha2VycyBzaG91bGQgcHJvdmlkZSBmaW5hbmNpYWwgaW5jZW50aXZlcywgaW52ZXN0IGluIHJlbmV3YWJsZSBpbmZyYXN0cnVjdHVyZSwgYW5kIGltcGxlbWVudCB0cmFpbmluZyBwcm9ncmFtcyB0byBzdXBwb3J0IGFkb3B0aW9uLiBDb2xsYWJvcmF0aXZlIGVmZm9ydHMgYXJlIGVzc2VudGlhbCBmb3Igc3VzdGFpbmFibGUgYWdyaWN1bHR1cmUgaW4gYXJpZCByZWdpb25zLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJpc3N1ZWREYXRlIjoiMjAyNS0wMy0wNSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkFncmljdWx0dXJlIiwiaXNzdWUiOjUsInZvbHVtZSI6MTUsImltcGFjdEZhY3RvciI6IjUuMjYifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjo1NTQsImxhc3RQYWdlIjo1NTR9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiTXVsdGlkaXNjaXBsaW5hcnkgRGlnaXRhbCBQdWJsaXNoaW5nIEluc3RpdHV0ZSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NDgsImNpdGVkQnlDb3VudCI6Mn19LCJxdW90ZSI6IjEuIEludHJvZHVjdGlvbiBUaGUgYWdyaWN1bHR1cmFsIHNlY3RvciwgYSBjb3JuZXJzdG9uZSBvZiBnbG9iYWwgZm9vZCBzZWN1cml0eSBhbmQgZWNvbm9taWMgZGV2ZWxvcG1lbnQsIGZhY2VzIG1vdW50aW5nIGNoYWxsZW5nZXMgaW4gdGhlIDIxc3QgY2VudHVyeS4gQ2xpbWF0ZSBjaGFuZ2UsIHJhcGlkIHBvcHVsYXRpb24gZ3Jvd3RoLCBhbmQgZHdpbmRsaW5nIG5hdHVyYWwgcmVzb3VyY2VzIGhhdmUgaW50ZW5zaWZpZWQgdGhlIG5lZWQgZm9yIHN1c3RhaW5hYmxlIGFncmljdWx0dXJhbCBwcmFjdGljZXMuIEFtb25nIHRoZXNlIGNoYWxsZW5nZXMsIHdhdGVyIHNjYXJjaXR5IGFuZCBlbmVyZ3kgaW5lZmZpY2llbmN5IHN0YW5kIG91dCBhcyBjcml0aWNhbCBiYXJyaWVycyB0byBwcm9kdWN0aXZpdHkgYW5kIHN1c3RhaW5hYmlsaXR5LiBHbG9iYWxseSwgYWdyaWN1bHR1cmUgYWNjb3VudHMgZm9yIGFwcHJveGltYXRlbHkgNzAlIG9mIGZyZXNod2F0ZXIgd2l0aGRyYXdhbHMgYW5kIGlzIG9uZSBvZiB0aGUgbGFyZ2VzdCBjb25zdW1lcnMgb2YgZW5lcmd5IGluIG1hbnkgcmVnaW9ucyBbIDEsIDJdLiBUcmFkaXRpb25hbCBpcnJpZ2F0aW9uIHByYWN0aWNlcywgb2Z0ZW4gaW5lZmZpY2llbnQgYW5kIHJlc291cmNlLWludGVuc2l2ZSwgZXhhY2VyYmF0ZSB0aGVzZSBpc3N1ZXMsIGxlYWRpbmcgdG8gd2F0ZXIgd2FzdGFnZSwgZW5lcmd5IG92ZXJ1c2UsIGFuZCByZWR1Y2VkIGNyb3AgeWllbGRzLiJ9XQ%3D%3D. These technologies represent a key advancement in modern irrigation, balancing coverage for large fields with improved water use efficiency amid global freshwater constraints where agriculture consumes ~70% of withdrawals (Daraz et al., 2025)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlVtYXIiLCJmYW1pbHkiOiJEYXJheiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiLFoHRlZmFuIiwiZmFtaWx5IjoiQm9qbmVjIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllvdW5hcyIsImZhbWlseSI6IktoYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMDc3LTA0NzIiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzOTAvYWdyaWN1bHR1cmUxNTA1MDU1NCIsIm9wZW5hbGV4IjoiVzQ0MDgyMTk4MTEiLCJleGEiOiJodHRwczovL3d3dy5tZHBpLmNvbS8yMDc3LTA0NzIvMTUvNS81NTQifSwidGl0bGUiOiJFbmVyZ3ktRWZmaWNpZW50IFNtYXJ0IElycmlnYXRpb24gVGVjaG5vbG9naWVzOiBBIFBhdGh3YXkgdG8gV2F0ZXIgYW5kIEVuZXJneSBTdXN0YWluYWJpbGl0eSBpbiBBZ3JpY3VsdHVyZSIsImFic3RyYWN0IjoiVGhlIGFncmljdWx0dXJhbCBzZWN0b3IgZmFjZXMgY2hhbGxlbmdlcyBzdWNoIGFzIHdhdGVyIHNjYXJjaXR5LCBlbmVyZ3kgaW5lZmZpY2llbmN5LCBhbmQgZGVjbGluaW5nIHByb2R1Y3Rpdml0eSwgcGFydGljdWxhcmx5IGluIGFyaWQgcmVnaW9ucy4gVHJhZGl0aW9uYWwgaXJyaWdhdGlvbiBtZXRob2RzIGNvbnRyaWJ1dGUgdG8gcmVzb3VyY2UgZGVwbGV0aW9uIGFuZCBlbnZpcm9ubWVudGFsIGltcGFjdHMuIFNvbGFyLXBvd2VyZWQgc21hcnQgaXJyaWdhdGlvbiBzeXN0ZW1zIGludGVncmF0ZSBwcmVjaXNpb24gaXJyaWdhdGlvbiB3aXRoIHJlbmV3YWJsZSBlbmVyZ3ksIGltcHJvdmluZyB3YXRlciB1c2UgYW5kIHByb2R1Y3Rpdml0eS4gSW4gUGFraXN0YW4sIHdoZXJlIGFncmljdWx0dXJlIGNvbnRyaWJ1dGVzIDE5JSBvZiBncm9zcyBkb21lc3RpYyBwcm9kdWN0IGFuZCBlbXBsb3lzIDQwJSBvZiB0aGUgd29ya2ZvcmNlLCB0aGVzZSBjaGFsbGVuZ2VzIGFyZSBzZXZlcmUsIGVzcGVjaWFsbHkgaW4gd2F0ZXItc2NhcmNlIGFyZWFzIGxpa2UgdGhlIENob2xpc3RhbiBEZXNlcnQuIFRoaXMgc3R1ZHkgZXhhbWluZXMgdGhlIGltcGFjdCBvZiBzb2xhci1wb3dlcmVkIHNtYXJ0IGlycmlnYXRpb24gb24gYWdyaWN1bHR1cmFsIHByb2R1Y3Rpdml0eSwgd2F0ZXIgY29uc2VydmF0aW9uLCBhbmQgZW5lcmd5IGVmZmljaWVuY3kgaW4gdGhlIENob2xpc3RhbiBEZXNlcnQuIFVzaW5nIGEgcXVhbnRpdGF0aXZlIGNyb3NzLXNlY3Rpb25hbCBkZXNpZ24sIGRhdGEgd2VyZSBjb2xsZWN0ZWQgZnJvbSAzODQgZmFybWVycyB2aWEgc3RydWN0dXJlZCBxdWVzdGlvbm5haXJlcy4gU3RhdGlzdGljYWwgYW5hbHlzZXMsIGluY2x1ZGluZyBtdWx0aXBsZSBsaW5lYXIgcmVncmVzc2lvbiwgcGFpcmVkIHNhbXBsZSB0LXRlc3RzLCBhbmQgU3RydWN0dXJhbCBFcXVhdGlvbiBNb2RlbGluZyAoU0VNKSwgd2VyZSBjb25kdWN0ZWQuIEZpbmRpbmdzIHNob3cgc2lnbmlmaWNhbnQgaW1wcm92ZW1lbnRzIGluIGNyb3AgeWllbGQgKGZyb20gMy4wIHRvIDQuOCB0b25zL2hlY3RhcmUpIGFuZCByZWR1Y3Rpb25zIGluIHdhdGVyIGFuZCBlbmVyZ3kgY29uc3VtcHRpb24uIFJlZ3Jlc3Npb24gYW5hbHlzaXMgaGlnaGxpZ2h0ZWQgc3Ryb25nIHBvc2l0aXZlIGVmZmVjdHMgb24geWllbGQgYW5kIGVmZmljaWVuY3ksIHdoaWxlIFNFTSBjb25maXJtZWQgcmVkdWNlZCBlbnZpcm9ubWVudGFsIGltcGFjdCBhbmQgb3BlcmF0aW9uYWwgY29zdHMuIFRoZSBzdHVkeSBjb25jbHVkZXMgdGhhdCBzb2xhci1wb3dlcmVkIGlycmlnYXRpb24gZW5oYW5jZXMgcHJvZHVjdGl2aXR5LCBjb25zZXJ2ZXMgcmVzb3VyY2VzLCBhbmQgcHJvbW90ZXMgc3VzdGFpbmFiaWxpdHkuIFBvbGljeW1ha2VycyBzaG91bGQgcHJvdmlkZSBmaW5hbmNpYWwgaW5jZW50aXZlcywgaW52ZXN0IGluIHJlbmV3YWJsZSBpbmZyYXN0cnVjdHVyZSwgYW5kIGltcGxlbWVudCB0cmFpbmluZyBwcm9ncmFtcyB0byBzdXBwb3J0IGFkb3B0aW9uLiBDb2xsYWJvcmF0aXZlIGVmZm9ydHMgYXJlIGVzc2VudGlhbCBmb3Igc3VzdGFpbmFibGUgYWdyaWN1bHR1cmUgaW4gYXJpZCByZWdpb25zLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJpc3N1ZWREYXRlIjoiMjAyNS0wMy0wNSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkFncmljdWx0dXJlIiwiaXNzdWUiOjUsInZvbHVtZSI6MTUsImltcGFjdEZhY3RvciI6IjUuMjYifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjo1NTQsImxhc3RQYWdlIjo1NTR9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiTXVsdGlkaXNjaXBsaW5hcnkgRGlnaXRhbCBQdWJsaXNoaW5nIEluc3RpdHV0ZSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NDgsImNpdGVkQnlDb3VudCI6Mn19LCJxdW90ZSI6IjEuIEludHJvZHVjdGlvbiBUaGUgYWdyaWN1bHR1cmFsIHNlY3RvciwgYSBjb3JuZXJzdG9uZSBvZiBnbG9iYWwgZm9vZCBzZWN1cml0eSBhbmQgZWNvbm9taWMgZGV2ZWxvcG1lbnQsIGZhY2VzIG1vdW50aW5nIGNoYWxsZW5nZXMgaW4gdGhlIDIxc3QgY2VudHVyeS4gQ2xpbWF0ZSBjaGFuZ2UsIHJhcGlkIHBvcHVsYXRpb24gZ3Jvd3RoLCBhbmQgZHdpbmRsaW5nIG5hdHVyYWwgcmVzb3VyY2VzIGhhdmUgaW50ZW5zaWZpZWQgdGhlIG5lZWQgZm9yIHN1c3RhaW5hYmxlIGFncmljdWx0dXJhbCBwcmFjdGljZXMuIEFtb25nIHRoZXNlIGNoYWxsZW5nZXMsIHdhdGVyIHNjYXJjaXR5IGFuZCBlbmVyZ3kgaW5lZmZpY2llbmN5IHN0YW5kIG91dCBhcyBjcml0aWNhbCBiYXJyaWVycyB0byBwcm9kdWN0aXZpdHkgYW5kIHN1c3RhaW5hYmlsaXR5LiBHbG9iYWxseSwgYWdyaWN1bHR1cmUgYWNjb3VudHMgZm9yIGFwcHJveGltYXRlbHkgNzAlIG9mIGZyZXNod2F0ZXIgd2l0aGRyYXdhbHMgYW5kIGlzIG9uZSBvZiB0aGUgbGFyZ2VzdCBjb25zdW1lcnMgb2YgZW5lcmd5IGluIG1hbnkgcmVnaW9ucyBbIDEsIDJdLiBUcmFkaXRpb25hbCBpcnJpZ2F0aW9uIHByYWN0aWNlcywgb2Z0ZW4gaW5lZmZpY2llbnQgYW5kIHJlc291cmNlLWludGVuc2l2ZSwgZXhhY2VyYmF0ZSB0aGVzZSBpc3N1ZXMsIGxlYWRpbmcgdG8gd2F0ZXIgd2FzdGFnZSwgZW5lcmd5IG92ZXJ1c2UsIGFuZCByZWR1Y2VkIGNyb3AgeWllbGRzLiJ9XQ%3D%3D.

### Subsurface Drip Irrigation

Subsurface drip irrigation buries drip lines 15-60 cm below soil surface, delivering water directly to root zones with efficiencies exceeding 90-95%, surpassing surface drip by eliminating evaporation and surface disruptions (Jumintono et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikp1bWludG9ubyIsImZhbWlseSI6Ikp1bWludG9ubyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJOdXJuYXdhdHkiLCJmYW1pbHkiOiJOdXJuYXdhdHkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTnVncm9obyBUcmlzbnUiLCJmYW1pbHkiOiJCcmF0YSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJEaW5pIiwiZmFtaWx5IjoiUmFobWlhdGkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSGFkaSIsImZhbWlseSI6IlN1c2lsbyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBLiBBLiIsImZhbWlseSI6Ikh1c3NlbiJ9fV0sImlkcyI6eyJpc3NuIjpbIjE0MjktNzQyNiIsIjIwODMtNDUzNSJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMjQ0MjUvandsZC4yMDI0LjE1MTc5OCIsIm9wZW5hbGV4IjoiVzQ0MDU0NzcxMDAifSwidGl0bGUiOiJDb21wYXJhdGl2ZSBhbmFseXNpcyBvZiBwcmVjaXNpb24sIGRlZmljaXQsIGFuZCBkcmlwIGlycmlnYXRpb24gZm9yIHdhdGVyIGVmZmljaWVuY3kgaW4gSXJhcSIsImFic3RyYWN0IjoiU3VzdGFpbmFibGUgaXJyaWdhdGlvbiBwcmFjdGljZXMgYXJlIGNyaXRpY2FsIGZvciBhZGRyZXNzaW5nIHRoZSBjaGFsbGVuZ2VzIG9mIHdhdGVyIHNjYXJjaXR5IGFuZCBmb29kIGluc2VjdXJpdHkgaW4gdGhlIGZhY2Ugb2YgY2xpbWF0ZSBjaGFuZ2UgYW5kIGdyb3dpbmcgZ2xvYmFsIGZvb2QgZGVtYW5kLiBUaGlzIHN0dWR5IGludmVzdGlnYXRlZCB0aGUgcG90ZW50aWFsIG9mIHByZWNpc2lvbiBpcnJpZ2F0aW9uIChQSSksIGRlZmljaXQgaXJyaWdhdGlvbiAoREkpLCBhbmQgZHJpcCBpcnJpZ2F0aW9uIChEUkkpIHRlY2huaXF1ZXMgdG8gZW5oYW5jZSBjcm9wIHByb2R1Y3Rpdml0eSBhbmQgd2F0ZXIgdXNlIGVmZmljaWVuY3kgKFdVRSkgY29tcGFyZWQgdG8gY29udmVudGlvbmFsIGZsb29kIGlycmlnYXRpb24gKENGSSkgaW4gdGhyZWUgZGlzdGluY3QgYWdyb2Vjb2xvZ2ljYWwgem9uZXMgb2YgSXJhcS4gRmllbGQgZXhwZXJpbWVudHMgd2VyZSBjb25kdWN0ZWQgdXNpbmcgYSByYW5kb21pc2VkIGNvbXBsZXRlIGJsb2NrIGRlc2lnbiB3aXRoIHdoZWF0LCBtYWl6ZSwgYW5kIHJpY2UgY3JvcHMuIFRoZSByZXN1bHRzIGRlbW9uc3RyYXRlZCB0aGF0IFBJIHNpZ25pZmljYW50bHkgaW5jcmVhc2VkIGNyb3AgZ3Jvd3RoIHBhcmFtZXRlcnMsIGdyYWluIHlpZWxkLCBhbmQgV1VFIGFjcm9zcyBhbGwgem9uZXMsIHdpdGggeWllbGQgaW1wcm92ZW1lbnRzIG9mIDMz4oCTMzglIGFuZCBXVUUgaW5jcmVhc2VzIG9mIDQ24oCTNTElIGluIGNvbnRyYXN0IHdpdGggQ0ZJLiBUaGUgREkgYW5kIERSSSB0cmVhdG1lbnRzIGFsc28gb3V0cGVyZm9ybWVkIENGSSwgYWxiZWl0IHRvIGEgbGVzc2VyIGV4dGVudC4gUmVtb3RlIHNlbnNpbmctZGVyaXZlZCB2ZWdldGF0aW9uIGluZGljZXMgc3Ryb25nbHkgY29ycmVsYXRlZCB3aXRoIGNyb3AgZ3Jvd3RoIHBhcmFtZXRlcnMgYW5kIHlpZWxkLCB3aGlsZSBoeWRyb2xvZ2ljYWwgbW9kZWxsaW5nIHJldmVhbGVkIHJlZHVjZWQgZXZhcG90cmFuc3BpcmF0aW9uIGFuZCBzdXJmYWNlIHJ1bm9mZiB1bmRlciB0aGUgUEkgdHJlYXRtZW50LiBUaGUgc3VzdGFpbmFibGUgaXJyaWdhdGlvbiBwcmFjdGljZXMgcmVzdWx0ZWQgaW4gc3Vic3RhbnRpYWwgd2F0ZXIgc2F2aW5ncyBvZiAyMOKAkzMwJSBjb21wYXJlZCB0byBDRkkuIFRoZXNlIGZpbmRpbmdzIGhpZ2hsaWdodCB0aGUgaW1wb3J0YW5jZSBvZiBhZG9wdGluZyBlZmZpY2llbnQgaXJyaWdhdGlvbiB0ZWNobmlxdWVzLCBhbG9uZyB3aXRoIGEgaG9saXN0aWMgYXBwcm9hY2ggZW5jb21wYXNzaW5nIHRlY2hub2xvZ2ljYWwgaW5ub3ZhdGlvbnMsIGNhcGFjaXR5IGJ1aWxkaW5nLCBhbmQgc3Rha2Vob2xkZXIgZW5nYWdlbWVudCwgdG8gcHJvbW90ZSB3YXRlci0gZWZmaWNpZW50IGFncmljdWx0dXJlIGFuZCBlbnN1cmUgZm9vZCBzZWN1cml0eSBpbiB3YXRlci1zY2FyY2UgcmVnaW9ucy4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjQ0MjUvandsZC4yMDI0LjE1MTc5OCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjI0NDI1L2p3bGQuMjAyNC4xNTE3OTgiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMi0xNCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkpvdXJuYWwgb2YgV2F0ZXIgYW5kIExhbmQgRGV2ZWxvcG1lbnQiLCJpbXBhY3RGYWN0b3IiOiIxLjQ2In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTMwLCJsYXN0UGFnZSI6MTM2fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkRlIEdydXl0ZXIgT3BlbiJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MzIsImNpdGVkQnlDb3VudCI6MX19LCJxdW90ZSI6IkluIHRoaXMgY29udGV4dCwgdGhlIGFkb3B0aW9uIG9mICBzdXN0YWluYWJsZSBpcnJpZ2F0aW9uIHByYWN0aWNlcyBoYXMgZW1lcmdlZCBhcyBhIGNyaXRpY2FsIHN0cmF0ZWd5ICBmb3IgZW5oYW5jaW5nIGZvb2Qgc2VjdXJpdHkgd2hpbGUgcHJvbW90aW5nIHdhdGVyIGNvbnNlcnZhdGlvbiAgaW4gYWdyaWN1bHR1cmFsIGxhbmRzY2FwZXMuICBJcnJpZ2F0aW9uIHBsYXlzIGEgdml0YWwgcm9sZSBpbiBnbG9iYWwgZm9vZCBwcm9kdWN0aW9uLCB3aXRoICBpcnJpZ2F0ZWQgYWdyaWN1bHR1cmUgYWNjb3VudGluZyBmb3IgYXBwcm94aW1hdGVseSA0MCUgb2YgdGhlICBmb29kIHN1cHBseSAoV3JhY2hpZW4gZGUsIFNjaHVsdHogYW5kIEdvbGksIDIwMjE7IFNjaG1pdHQsIFJvc2EgIGFuZCBEYWlseSwgMjAyMikuIEhvd2V2ZXIsIHRyYWRpdGlvbmFsIGlycmlnYXRpb24gbWV0aG9kcywgc3VjaCBhcyAgZmxvb2QgYW5kIGZ1cnJvdyBpcnJpZ2F0aW9uLCBhcmUgb2Z0ZW4gaW5lZmZpY2llbnQsIGxlYWRpbmcgdG8gIHN1YnN0YW50aWFsIHdhdGVyIGxvc3NlcyB0aHJvdWdoIGV2YXBvcmF0aW9uLCBydW5vZmYsIGFuZCBkZWVwICBwZXJjb2xhdGlvbiAoQW1ib21zYSwgMjAyMCkuIE1vcmVvdmVyLCB0aGUgZXhjZXNzaXZlIGFwcGxpY2EtdGlvbiBvZiB3YXRlciBjYW4gcmVzdWx0IGluIHNvaWwgc2FsaW5pc2F0aW9uLCBudXRyaWVudCBsZWFjaGluZywgYW5kICBncm91bmR3YXRlciBkZXBsZXRpb24sIGZ1cnRoZXIgY29tcHJvbWlzaW5nIHRoZSBsb25nLXRlcm0gIHN1c3RhaW5hYmlsaXR5IG9mIGFncmljdWx0dXJhbCBzeXN0ZW1zIChZaW4gZXQgYWwuLCAyMDIyKS4gVG8gYWRkcmVzcyAgdGhlc2UgY2hhbGxlbmdlcywgcmVzZWFyY2hlcnMgYW5kIHByYWN0aXRpb25lcnMgaGF2ZSBiZWVuIGV4cGxvci1pbmcgaW5ub3ZhdGl2ZSBpcnJpZ2F0aW9uIHRlY2huaXF1ZXMgYW5kIHdhdGVyIG1hbmFnZW1lbnQgIHN0cmF0ZWdpZXMgdGhhdCBjYW4gb3B0aW1pc2Ugd2F0ZXIgdXNlIGVmZmljaWVuY3kgKFdVRSkgd2hpbGUgIG1haW50YWluaW5nIG9yIGVuaGFuY2luZyBjcm9wIHByb2R1Y3Rpdml0eS4gIFByZWNpc2lvbiBpcnJpZ2F0aW9uIChQSSkgb2ZmZXJzIGEgcHJvbWlzaW5nIHNvbHV0aW9uLCAgaW52b2x2aW5nIHRoZSB0YXJnZXRlZCBhcHBsaWNhdGlvbiBvZiB3YXRlciBzcGVjaWZpY2FsbHkgdG8gdGhlICBuZWVkcyBvZiBpbmRpdmlkdWFsIHBsYW50cyBvciBjcm9wIHpvbmVzIChaaW5rZXJuYWdlbCBldCBhbC4sIDIwMjApLiAgIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyMi44ODY3MzU5MTYxMzc2OTUsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzQ2MjIxOTIzODI4MTI1LCJoZWlnaHQiOjEuNDUwNTQxMDE5NDM5Njk3M319LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI0LjM1MTEyMzgwOTgxNDQ1MywibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTI3MTYwNjQ0NSwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI1LjcyNDk4NzAzMDAyOTI5NywibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTY1MzA3NjE3MiwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI3LjA5ODg1MDI1MDI0NDE0LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjE2LjQxODE2MTM5MjIxMTkxNCwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI4LjQ3MjcwOTY1NTc2MTcyLCJsZWZ0IjoxMi4zODA4ODAzNTU4MzQ5NjEsIndpZHRoIjozNy4wMTIxODQxNDMwNjY0MDYsImhlaWdodCI6MS4zNjAwMTQ3OTYyNTcwMTl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyOS44NDY1NzI4NzU5NzY1NjIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQwNjQ4NjUxMTIzMDUsImhlaWdodCI6MS4zNjAwMTY3MDM2MDU2NTE5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzEuMjE0MDM2OTQxNTI4MzIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQ0ODU2MjYyMjA3MDMsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzIuNTg3OTAyMDY5MDkxOCwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDYwNTc4OTE4NDU3LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMzLjk2MTc2MTQ3NDYwOTM3NSwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDA0ODQ2MTkxNDA2MjUsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzUuMzM1NjI0Njk0ODI0MjIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzM5NzQ0NTY3ODcxMDk0LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjM2LjcwOTQ4NzkxNTAzOTA2LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjAwMTU5MDcyODc1OTc2NiwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC4wODMzNDczMjA1NTY2NCwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTY1MzA3NjE3MiwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOS40NTcyMTA1NDA3NzE0ODQsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQzMDQ4MDk1NzAzMTI1LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQwLjc0MDU0NzE4MDE3NTc4LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjM0NTQ1NTE2OTY3NzczNCwiaGVpZ2h0IjoxLjQ1MDU0MjgwNzU3OTA0MDV9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Mi4yMDQ5MzMxNjY1MDM5MDYsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMDAzMjUwMTIyMDcwMzEsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDMuNTc4Nzk2Mzg2NzE4NzUsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzM3OTIxMTQyNTc4MTI1LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQ0Ljg2MjEzMzAyNjEyMzA1LCJsZWZ0Ijo5LjA0NzYwMTY5OTgyOTEwMiwid2lkdGgiOjQwLjM0NTMxNDAyNTg3ODkwNiwiaGVpZ2h0IjoxLjQ1MDU0MjgwNzU3OTA0MDV9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ni4zMjY1MTkwMTI0NTExNywibGVmdCI6OS4wNDc2MDE2OTk4MjkxMDIsIndpZHRoIjoyNy45Njc0NDcyODA4ODM3OSwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ny43MDAzODIyMzI2NjYwMTYsImxlZnQiOjEyLjM4MDg3ODQ0ODQ4NjMyOCwid2lkdGgiOjM3LjAzNDc4NjIyNDM2NTIzNCwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0OS4wNzQyNDU0NTI4ODA4NiwibGVmdCI6OS4wNDc2MDE2OTk4MjkxMDIsIndpZHRoIjo0MC4zNDA2NDQ4MzY0MjU3OCwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1MC4zNTc1ODIwOTIyODUxNTYsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzQ0ODUyNDQ3NTA5NzY2LCJoZWlnaHQiOjEuNDUwNTQyODA3NTc5MDQwNX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEzMSJ9fV0%3D. This method addresses precision needs in water-scarce regions, optimizing WUE while combating issues like salinization from traditional over-application (Jumintono et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikp1bWludG9ubyIsImZhbWlseSI6Ikp1bWludG9ubyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJOdXJuYXdhdHkiLCJmYW1pbHkiOiJOdXJuYXdhdHkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTnVncm9obyBUcmlzbnUiLCJmYW1pbHkiOiJCcmF0YSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJEaW5pIiwiZmFtaWx5IjoiUmFobWlhdGkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSGFkaSIsImZhbWlseSI6IlN1c2lsbyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBLiBBLiIsImZhbWlseSI6Ikh1c3NlbiJ9fV0sImlkcyI6eyJpc3NuIjpbIjE0MjktNzQyNiIsIjIwODMtNDUzNSJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMjQ0MjUvandsZC4yMDI0LjE1MTc5OCIsIm9wZW5hbGV4IjoiVzQ0MDU0NzcxMDAifSwidGl0bGUiOiJDb21wYXJhdGl2ZSBhbmFseXNpcyBvZiBwcmVjaXNpb24sIGRlZmljaXQsIGFuZCBkcmlwIGlycmlnYXRpb24gZm9yIHdhdGVyIGVmZmljaWVuY3kgaW4gSXJhcSIsImFic3RyYWN0IjoiU3VzdGFpbmFibGUgaXJyaWdhdGlvbiBwcmFjdGljZXMgYXJlIGNyaXRpY2FsIGZvciBhZGRyZXNzaW5nIHRoZSBjaGFsbGVuZ2VzIG9mIHdhdGVyIHNjYXJjaXR5IGFuZCBmb29kIGluc2VjdXJpdHkgaW4gdGhlIGZhY2Ugb2YgY2xpbWF0ZSBjaGFuZ2UgYW5kIGdyb3dpbmcgZ2xvYmFsIGZvb2QgZGVtYW5kLiBUaGlzIHN0dWR5IGludmVzdGlnYXRlZCB0aGUgcG90ZW50aWFsIG9mIHByZWNpc2lvbiBpcnJpZ2F0aW9uIChQSSksIGRlZmljaXQgaXJyaWdhdGlvbiAoREkpLCBhbmQgZHJpcCBpcnJpZ2F0aW9uIChEUkkpIHRlY2huaXF1ZXMgdG8gZW5oYW5jZSBjcm9wIHByb2R1Y3Rpdml0eSBhbmQgd2F0ZXIgdXNlIGVmZmljaWVuY3kgKFdVRSkgY29tcGFyZWQgdG8gY29udmVudGlvbmFsIGZsb29kIGlycmlnYXRpb24gKENGSSkgaW4gdGhyZWUgZGlzdGluY3QgYWdyb2Vjb2xvZ2ljYWwgem9uZXMgb2YgSXJhcS4gRmllbGQgZXhwZXJpbWVudHMgd2VyZSBjb25kdWN0ZWQgdXNpbmcgYSByYW5kb21pc2VkIGNvbXBsZXRlIGJsb2NrIGRlc2lnbiB3aXRoIHdoZWF0LCBtYWl6ZSwgYW5kIHJpY2UgY3JvcHMuIFRoZSByZXN1bHRzIGRlbW9uc3RyYXRlZCB0aGF0IFBJIHNpZ25pZmljYW50bHkgaW5jcmVhc2VkIGNyb3AgZ3Jvd3RoIHBhcmFtZXRlcnMsIGdyYWluIHlpZWxkLCBhbmQgV1VFIGFjcm9zcyBhbGwgem9uZXMsIHdpdGggeWllbGQgaW1wcm92ZW1lbnRzIG9mIDMz4oCTMzglIGFuZCBXVUUgaW5jcmVhc2VzIG9mIDQ24oCTNTElIGluIGNvbnRyYXN0IHdpdGggQ0ZJLiBUaGUgREkgYW5kIERSSSB0cmVhdG1lbnRzIGFsc28gb3V0cGVyZm9ybWVkIENGSSwgYWxiZWl0IHRvIGEgbGVzc2VyIGV4dGVudC4gUmVtb3RlIHNlbnNpbmctZGVyaXZlZCB2ZWdldGF0aW9uIGluZGljZXMgc3Ryb25nbHkgY29ycmVsYXRlZCB3aXRoIGNyb3AgZ3Jvd3RoIHBhcmFtZXRlcnMgYW5kIHlpZWxkLCB3aGlsZSBoeWRyb2xvZ2ljYWwgbW9kZWxsaW5nIHJldmVhbGVkIHJlZHVjZWQgZXZhcG90cmFuc3BpcmF0aW9uIGFuZCBzdXJmYWNlIHJ1bm9mZiB1bmRlciB0aGUgUEkgdHJlYXRtZW50LiBUaGUgc3VzdGFpbmFibGUgaXJyaWdhdGlvbiBwcmFjdGljZXMgcmVzdWx0ZWQgaW4gc3Vic3RhbnRpYWwgd2F0ZXIgc2F2aW5ncyBvZiAyMOKAkzMwJSBjb21wYXJlZCB0byBDRkkuIFRoZXNlIGZpbmRpbmdzIGhpZ2hsaWdodCB0aGUgaW1wb3J0YW5jZSBvZiBhZG9wdGluZyBlZmZpY2llbnQgaXJyaWdhdGlvbiB0ZWNobmlxdWVzLCBhbG9uZyB3aXRoIGEgaG9saXN0aWMgYXBwcm9hY2ggZW5jb21wYXNzaW5nIHRlY2hub2xvZ2ljYWwgaW5ub3ZhdGlvbnMsIGNhcGFjaXR5IGJ1aWxkaW5nLCBhbmQgc3Rha2Vob2xkZXIgZW5nYWdlbWVudCwgdG8gcHJvbW90ZSB3YXRlci0gZWZmaWNpZW50IGFncmljdWx0dXJlIGFuZCBlbnN1cmUgZm9vZCBzZWN1cml0eSBpbiB3YXRlci1zY2FyY2UgcmVnaW9ucy4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjQ0MjUvandsZC4yMDI0LjE1MTc5OCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjI0NDI1L2p3bGQuMjAyNC4xNTE3OTgiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMi0xNCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkpvdXJuYWwgb2YgV2F0ZXIgYW5kIExhbmQgRGV2ZWxvcG1lbnQiLCJpbXBhY3RGYWN0b3IiOiIxLjQ2In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTMwLCJsYXN0UGFnZSI6MTM2fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkRlIEdydXl0ZXIgT3BlbiJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MzIsImNpdGVkQnlDb3VudCI6MX19LCJxdW90ZSI6IkluIHRoaXMgY29udGV4dCwgdGhlIGFkb3B0aW9uIG9mICBzdXN0YWluYWJsZSBpcnJpZ2F0aW9uIHByYWN0aWNlcyBoYXMgZW1lcmdlZCBhcyBhIGNyaXRpY2FsIHN0cmF0ZWd5ICBmb3IgZW5oYW5jaW5nIGZvb2Qgc2VjdXJpdHkgd2hpbGUgcHJvbW90aW5nIHdhdGVyIGNvbnNlcnZhdGlvbiAgaW4gYWdyaWN1bHR1cmFsIGxhbmRzY2FwZXMuICBJcnJpZ2F0aW9uIHBsYXlzIGEgdml0YWwgcm9sZSBpbiBnbG9iYWwgZm9vZCBwcm9kdWN0aW9uLCB3aXRoICBpcnJpZ2F0ZWQgYWdyaWN1bHR1cmUgYWNjb3VudGluZyBmb3IgYXBwcm94aW1hdGVseSA0MCUgb2YgdGhlICBmb29kIHN1cHBseSAoV3JhY2hpZW4gZGUsIFNjaHVsdHogYW5kIEdvbGksIDIwMjE7IFNjaG1pdHQsIFJvc2EgIGFuZCBEYWlseSwgMjAyMikuIEhvd2V2ZXIsIHRyYWRpdGlvbmFsIGlycmlnYXRpb24gbWV0aG9kcywgc3VjaCBhcyAgZmxvb2QgYW5kIGZ1cnJvdyBpcnJpZ2F0aW9uLCBhcmUgb2Z0ZW4gaW5lZmZpY2llbnQsIGxlYWRpbmcgdG8gIHN1YnN0YW50aWFsIHdhdGVyIGxvc3NlcyB0aHJvdWdoIGV2YXBvcmF0aW9uLCBydW5vZmYsIGFuZCBkZWVwICBwZXJjb2xhdGlvbiAoQW1ib21zYSwgMjAyMCkuIE1vcmVvdmVyLCB0aGUgZXhjZXNzaXZlIGFwcGxpY2EtdGlvbiBvZiB3YXRlciBjYW4gcmVzdWx0IGluIHNvaWwgc2FsaW5pc2F0aW9uLCBudXRyaWVudCBsZWFjaGluZywgYW5kICBncm91bmR3YXRlciBkZXBsZXRpb24sIGZ1cnRoZXIgY29tcHJvbWlzaW5nIHRoZSBsb25nLXRlcm0gIHN1c3RhaW5hYmlsaXR5IG9mIGFncmljdWx0dXJhbCBzeXN0ZW1zIChZaW4gZXQgYWwuLCAyMDIyKS4gVG8gYWRkcmVzcyAgdGhlc2UgY2hhbGxlbmdlcywgcmVzZWFyY2hlcnMgYW5kIHByYWN0aXRpb25lcnMgaGF2ZSBiZWVuIGV4cGxvci1pbmcgaW5ub3ZhdGl2ZSBpcnJpZ2F0aW9uIHRlY2huaXF1ZXMgYW5kIHdhdGVyIG1hbmFnZW1lbnQgIHN0cmF0ZWdpZXMgdGhhdCBjYW4gb3B0aW1pc2Ugd2F0ZXIgdXNlIGVmZmljaWVuY3kgKFdVRSkgd2hpbGUgIG1haW50YWluaW5nIG9yIGVuaGFuY2luZyBjcm9wIHByb2R1Y3Rpdml0eS4gIFByZWNpc2lvbiBpcnJpZ2F0aW9uIChQSSkgb2ZmZXJzIGEgcHJvbWlzaW5nIHNvbHV0aW9uLCAgaW52b2x2aW5nIHRoZSB0YXJnZXRlZCBhcHBsaWNhdGlvbiBvZiB3YXRlciBzcGVjaWZpY2FsbHkgdG8gdGhlICBuZWVkcyBvZiBpbmRpdmlkdWFsIHBsYW50cyBvciBjcm9wIHpvbmVzIChaaW5rZXJuYWdlbCBldCBhbC4sIDIwMjApLiAgIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyMi44ODY3MzU5MTYxMzc2OTUsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzQ2MjIxOTIzODI4MTI1LCJoZWlnaHQiOjEuNDUwNTQxMDE5NDM5Njk3M319LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI0LjM1MTEyMzgwOTgxNDQ1MywibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTI3MTYwNjQ0NSwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI1LjcyNDk4NzAzMDAyOTI5NywibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTY1MzA3NjE3MiwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI3LjA5ODg1MDI1MDI0NDE0LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjE2LjQxODE2MTM5MjIxMTkxNCwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI4LjQ3MjcwOTY1NTc2MTcyLCJsZWZ0IjoxMi4zODA4ODAzNTU4MzQ5NjEsIndpZHRoIjozNy4wMTIxODQxNDMwNjY0MDYsImhlaWdodCI6MS4zNjAwMTQ3OTYyNTcwMTl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyOS44NDY1NzI4NzU5NzY1NjIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQwNjQ4NjUxMTIzMDUsImhlaWdodCI6MS4zNjAwMTY3MDM2MDU2NTE5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzEuMjE0MDM2OTQxNTI4MzIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQ0ODU2MjYyMjA3MDMsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzIuNTg3OTAyMDY5MDkxOCwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDYwNTc4OTE4NDU3LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMzLjk2MTc2MTQ3NDYwOTM3NSwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDA0ODQ2MTkxNDA2MjUsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzUuMzM1NjI0Njk0ODI0MjIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzM5NzQ0NTY3ODcxMDk0LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjM2LjcwOTQ4NzkxNTAzOTA2LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjAwMTU5MDcyODc1OTc2NiwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC4wODMzNDczMjA1NTY2NCwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTY1MzA3NjE3MiwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOS40NTcyMTA1NDA3NzE0ODQsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQzMDQ4MDk1NzAzMTI1LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQwLjc0MDU0NzE4MDE3NTc4LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjM0NTQ1NTE2OTY3NzczNCwiaGVpZ2h0IjoxLjQ1MDU0MjgwNzU3OTA0MDV9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Mi4yMDQ5MzMxNjY1MDM5MDYsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMDAzMjUwMTIyMDcwMzEsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDMuNTc4Nzk2Mzg2NzE4NzUsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzM3OTIxMTQyNTc4MTI1LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQ0Ljg2MjEzMzAyNjEyMzA1LCJsZWZ0Ijo5LjA0NzYwMTY5OTgyOTEwMiwid2lkdGgiOjQwLjM0NTMxNDAyNTg3ODkwNiwiaGVpZ2h0IjoxLjQ1MDU0MjgwNzU3OTA0MDV9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ni4zMjY1MTkwMTI0NTExNywibGVmdCI6OS4wNDc2MDE2OTk4MjkxMDIsIndpZHRoIjoyNy45Njc0NDcyODA4ODM3OSwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ny43MDAzODIyMzI2NjYwMTYsImxlZnQiOjEyLjM4MDg3ODQ0ODQ4NjMyOCwid2lkdGgiOjM3LjAzNDc4NjIyNDM2NTIzNCwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0OS4wNzQyNDU0NTI4ODA4NiwibGVmdCI6OS4wNDc2MDE2OTk4MjkxMDIsIndpZHRoIjo0MC4zNDA2NDQ4MzY0MjU3OCwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1MC4zNTc1ODIwOTIyODUxNTYsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzQ0ODUyNDQ3NTA5NzY2LCJoZWlnaHQiOjEuNDUwNTQyODA3NTc5MDQwNX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEzMSJ9fV0%3D.

Figure 5: The IoT-Based Smart Irrigation Architecture

### Table 3: Comparison of Modern Irrigation Technologies

| Technology | Application Efficiency | Energy Demand | Best For |
| --- | --- | --- | --- |
| Drip Irrigation | 90–95% | Low | Targeted root zone delivery |
| Sprinkler | 70–85% | High | Large field-scale distribution |
| Subsurface Drip | High (>90%) | Low | Minimizing surface wastage |
| Center Pivot | Moderate/High | High | Large circular fields |
| Micro-sprayers | 85–90% | Moderate | Orchards and specific zones |
| Smart Sensors | High | Low | Real-time monitoring/scheduling |
| IoT-Based AI | Very High | Moderate | Adaptive, automated scheduling |
| Deficit Irrigation | High | Low | Areas with severe water scarcity |

Principles of Operation

Sprinkler systems propel water via pressurized pipes to nozzles that create spray patterns, while center-pivot systems use a rotating boom with emitters spanning hundreds of acres. Pumps, filters, and regulators maintain optimal pressure (3-5 bars), enabling uniform application and integration with fertigation (Daraz et al., 2025)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlVtYXIiLCJmYW1pbHkiOiJEYXJheiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiLFoHRlZmFuIiwiZmFtaWx5IjoiQm9qbmVjIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllvdW5hcyIsImZhbWlseSI6IktoYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMDc3LTA0NzIiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzOTAvYWdyaWN1bHR1cmUxNTA1MDU1NCIsIm9wZW5hbGV4IjoiVzQ0MDgyMTk4MTEiLCJleGEiOiJodHRwczovL3d3dy5tZHBpLmNvbS8yMDc3LTA0NzIvMTUvNS81NTQifSwidGl0bGUiOiJFbmVyZ3ktRWZmaWNpZW50IFNtYXJ0IElycmlnYXRpb24gVGVjaG5vbG9naWVzOiBBIFBhdGh3YXkgdG8gV2F0ZXIgYW5kIEVuZXJneSBTdXN0YWluYWJpbGl0eSBpbiBBZ3JpY3VsdHVyZSIsImFic3RyYWN0IjoiVGhlIGFncmljdWx0dXJhbCBzZWN0b3IgZmFjZXMgY2hhbGxlbmdlcyBzdWNoIGFzIHdhdGVyIHNjYXJjaXR5LCBlbmVyZ3kgaW5lZmZpY2llbmN5LCBhbmQgZGVjbGluaW5nIHByb2R1Y3Rpdml0eSwgcGFydGljdWxhcmx5IGluIGFyaWQgcmVnaW9ucy4gVHJhZGl0aW9uYWwgaXJyaWdhdGlvbiBtZXRob2RzIGNvbnRyaWJ1dGUgdG8gcmVzb3VyY2UgZGVwbGV0aW9uIGFuZCBlbnZpcm9ubWVudGFsIGltcGFjdHMuIFNvbGFyLXBvd2VyZWQgc21hcnQgaXJyaWdhdGlvbiBzeXN0ZW1zIGludGVncmF0ZSBwcmVjaXNpb24gaXJyaWdhdGlvbiB3aXRoIHJlbmV3YWJsZSBlbmVyZ3ksIGltcHJvdmluZyB3YXRlciB1c2UgYW5kIHByb2R1Y3Rpdml0eS4gSW4gUGFraXN0YW4sIHdoZXJlIGFncmljdWx0dXJlIGNvbnRyaWJ1dGVzIDE5JSBvZiBncm9zcyBkb21lc3RpYyBwcm9kdWN0IGFuZCBlbXBsb3lzIDQwJSBvZiB0aGUgd29ya2ZvcmNlLCB0aGVzZSBjaGFsbGVuZ2VzIGFyZSBzZXZlcmUsIGVzcGVjaWFsbHkgaW4gd2F0ZXItc2NhcmNlIGFyZWFzIGxpa2UgdGhlIENob2xpc3RhbiBEZXNlcnQuIFRoaXMgc3R1ZHkgZXhhbWluZXMgdGhlIGltcGFjdCBvZiBzb2xhci1wb3dlcmVkIHNtYXJ0IGlycmlnYXRpb24gb24gYWdyaWN1bHR1cmFsIHByb2R1Y3Rpdml0eSwgd2F0ZXIgY29uc2VydmF0aW9uLCBhbmQgZW5lcmd5IGVmZmljaWVuY3kgaW4gdGhlIENob2xpc3RhbiBEZXNlcnQuIFVzaW5nIGEgcXVhbnRpdGF0aXZlIGNyb3NzLXNlY3Rpb25hbCBkZXNpZ24sIGRhdGEgd2VyZSBjb2xsZWN0ZWQgZnJvbSAzODQgZmFybWVycyB2aWEgc3RydWN0dXJlZCBxdWVzdGlvbm5haXJlcy4gU3RhdGlzdGljYWwgYW5hbHlzZXMsIGluY2x1ZGluZyBtdWx0aXBsZSBsaW5lYXIgcmVncmVzc2lvbiwgcGFpcmVkIHNhbXBsZSB0LXRlc3RzLCBhbmQgU3RydWN0dXJhbCBFcXVhdGlvbiBNb2RlbGluZyAoU0VNKSwgd2VyZSBjb25kdWN0ZWQuIEZpbmRpbmdzIHNob3cgc2lnbmlmaWNhbnQgaW1wcm92ZW1lbnRzIGluIGNyb3AgeWllbGQgKGZyb20gMy4wIHRvIDQuOCB0b25zL2hlY3RhcmUpIGFuZCByZWR1Y3Rpb25zIGluIHdhdGVyIGFuZCBlbmVyZ3kgY29uc3VtcHRpb24uIFJlZ3Jlc3Npb24gYW5hbHlzaXMgaGlnaGxpZ2h0ZWQgc3Ryb25nIHBvc2l0aXZlIGVmZmVjdHMgb24geWllbGQgYW5kIGVmZmljaWVuY3ksIHdoaWxlIFNFTSBjb25maXJtZWQgcmVkdWNlZCBlbnZpcm9ubWVudGFsIGltcGFjdCBhbmQgb3BlcmF0aW9uYWwgY29zdHMuIFRoZSBzdHVkeSBjb25jbHVkZXMgdGhhdCBzb2xhci1wb3dlcmVkIGlycmlnYXRpb24gZW5oYW5jZXMgcHJvZHVjdGl2aXR5LCBjb25zZXJ2ZXMgcmVzb3VyY2VzLCBhbmQgcHJvbW90ZXMgc3VzdGFpbmFiaWxpdHkuIFBvbGljeW1ha2VycyBzaG91bGQgcHJvdmlkZSBmaW5hbmNpYWwgaW5jZW50aXZlcywgaW52ZXN0IGluIHJlbmV3YWJsZSBpbmZyYXN0cnVjdHVyZSwgYW5kIGltcGxlbWVudCB0cmFpbmluZyBwcm9ncmFtcyB0byBzdXBwb3J0IGFkb3B0aW9uLiBDb2xsYWJvcmF0aXZlIGVmZm9ydHMgYXJlIGVzc2VudGlhbCBmb3Igc3VzdGFpbmFibGUgYWdyaWN1bHR1cmUgaW4gYXJpZCByZWdpb25zLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJpc3N1ZWREYXRlIjoiMjAyNS0wMy0wNSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkFncmljdWx0dXJlIiwiaXNzdWUiOjUsInZvbHVtZSI6MTUsImltcGFjdEZhY3RvciI6IjUuMjYifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjo1NTQsImxhc3RQYWdlIjo1NTR9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiTXVsdGlkaXNjaXBsaW5hcnkgRGlnaXRhbCBQdWJsaXNoaW5nIEluc3RpdHV0ZSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NDgsImNpdGVkQnlDb3VudCI6Mn19LCJxdW90ZSI6IjEuIEludHJvZHVjdGlvbiBUaGUgYWdyaWN1bHR1cmFsIHNlY3RvciwgYSBjb3JuZXJzdG9uZSBvZiBnbG9iYWwgZm9vZCBzZWN1cml0eSBhbmQgZWNvbm9taWMgZGV2ZWxvcG1lbnQsIGZhY2VzIG1vdW50aW5nIGNoYWxsZW5nZXMgaW4gdGhlIDIxc3QgY2VudHVyeS4gQ2xpbWF0ZSBjaGFuZ2UsIHJhcGlkIHBvcHVsYXRpb24gZ3Jvd3RoLCBhbmQgZHdpbmRsaW5nIG5hdHVyYWwgcmVzb3VyY2VzIGhhdmUgaW50ZW5zaWZpZWQgdGhlIG5lZWQgZm9yIHN1c3RhaW5hYmxlIGFncmljdWx0dXJhbCBwcmFjdGljZXMuIEFtb25nIHRoZXNlIGNoYWxsZW5nZXMsIHdhdGVyIHNjYXJjaXR5IGFuZCBlbmVyZ3kgaW5lZmZpY2llbmN5IHN0YW5kIG91dCBhcyBjcml0aWNhbCBiYXJyaWVycyB0byBwcm9kdWN0aXZpdHkgYW5kIHN1c3RhaW5hYmlsaXR5LiBHbG9iYWxseSwgYWdyaWN1bHR1cmUgYWNjb3VudHMgZm9yIGFwcHJveGltYXRlbHkgNzAlIG9mIGZyZXNod2F0ZXIgd2l0aGRyYXdhbHMgYW5kIGlzIG9uZSBvZiB0aGUgbGFyZ2VzdCBjb25zdW1lcnMgb2YgZW5lcmd5IGluIG1hbnkgcmVnaW9ucyBbIDEsIDJdLiBUcmFkaXRpb25hbCBpcnJpZ2F0aW9uIHByYWN0aWNlcywgb2Z0ZW4gaW5lZmZpY2llbnQgYW5kIHJlc291cmNlLWludGVuc2l2ZSwgZXhhY2VyYmF0ZSB0aGVzZSBpc3N1ZXMsIGxlYWRpbmcgdG8gd2F0ZXIgd2FzdGFnZSwgZW5lcmd5IG92ZXJ1c2UsIGFuZCByZWR1Y2VkIGNyb3AgeWllbGRzLiJ9XQ%3D%3D.

Advantages and Efficiency Gains

- Water savings: 30-50% reduction versus surface irrigation through controlled droplet size minimizing evaporation (Daraz et al., 2025)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlVtYXIiLCJmYW1pbHkiOiJEYXJheiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiLFoHRlZmFuIiwiZmFtaWx5IjoiQm9qbmVjIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllvdW5hcyIsImZhbWlseSI6IktoYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMDc3LTA0NzIiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzOTAvYWdyaWN1bHR1cmUxNTA1MDU1NCIsIm9wZW5hbGV4IjoiVzQ0MDgyMTk4MTEiLCJleGEiOiJodHRwczovL3d3dy5tZHBpLmNvbS8yMDc3LTA0NzIvMTUvNS81NTQifSwidGl0bGUiOiJFbmVyZ3ktRWZmaWNpZW50IFNtYXJ0IElycmlnYXRpb24gVGVjaG5vbG9naWVzOiBBIFBhdGh3YXkgdG8gV2F0ZXIgYW5kIEVuZXJneSBTdXN0YWluYWJpbGl0eSBpbiBBZ3JpY3VsdHVyZSIsImFic3RyYWN0IjoiVGhlIGFncmljdWx0dXJhbCBzZWN0b3IgZmFjZXMgY2hhbGxlbmdlcyBzdWNoIGFzIHdhdGVyIHNjYXJjaXR5LCBlbmVyZ3kgaW5lZmZpY2llbmN5LCBhbmQgZGVjbGluaW5nIHByb2R1Y3Rpdml0eSwgcGFydGljdWxhcmx5IGluIGFyaWQgcmVnaW9ucy4gVHJhZGl0aW9uYWwgaXJyaWdhdGlvbiBtZXRob2RzIGNvbnRyaWJ1dGUgdG8gcmVzb3VyY2UgZGVwbGV0aW9uIGFuZCBlbnZpcm9ubWVudGFsIGltcGFjdHMuIFNvbGFyLXBvd2VyZWQgc21hcnQgaXJyaWdhdGlvbiBzeXN0ZW1zIGludGVncmF0ZSBwcmVjaXNpb24gaXJyaWdhdGlvbiB3aXRoIHJlbmV3YWJsZSBlbmVyZ3ksIGltcHJvdmluZyB3YXRlciB1c2UgYW5kIHByb2R1Y3Rpdml0eS4gSW4gUGFraXN0YW4sIHdoZXJlIGFncmljdWx0dXJlIGNvbnRyaWJ1dGVzIDE5JSBvZiBncm9zcyBkb21lc3RpYyBwcm9kdWN0IGFuZCBlbXBsb3lzIDQwJSBvZiB0aGUgd29ya2ZvcmNlLCB0aGVzZSBjaGFsbGVuZ2VzIGFyZSBzZXZlcmUsIGVzcGVjaWFsbHkgaW4gd2F0ZXItc2NhcmNlIGFyZWFzIGxpa2UgdGhlIENob2xpc3RhbiBEZXNlcnQuIFRoaXMgc3R1ZHkgZXhhbWluZXMgdGhlIGltcGFjdCBvZiBzb2xhci1wb3dlcmVkIHNtYXJ0IGlycmlnYXRpb24gb24gYWdyaWN1bHR1cmFsIHByb2R1Y3Rpdml0eSwgd2F0ZXIgY29uc2VydmF0aW9uLCBhbmQgZW5lcmd5IGVmZmljaWVuY3kgaW4gdGhlIENob2xpc3RhbiBEZXNlcnQuIFVzaW5nIGEgcXVhbnRpdGF0aXZlIGNyb3NzLXNlY3Rpb25hbCBkZXNpZ24sIGRhdGEgd2VyZSBjb2xsZWN0ZWQgZnJvbSAzODQgZmFybWVycyB2aWEgc3RydWN0dXJlZCBxdWVzdGlvbm5haXJlcy4gU3RhdGlzdGljYWwgYW5hbHlzZXMsIGluY2x1ZGluZyBtdWx0aXBsZSBsaW5lYXIgcmVncmVzc2lvbiwgcGFpcmVkIHNhbXBsZSB0LXRlc3RzLCBhbmQgU3RydWN0dXJhbCBFcXVhdGlvbiBNb2RlbGluZyAoU0VNKSwgd2VyZSBjb25kdWN0ZWQuIEZpbmRpbmdzIHNob3cgc2lnbmlmaWNhbnQgaW1wcm92ZW1lbnRzIGluIGNyb3AgeWllbGQgKGZyb20gMy4wIHRvIDQuOCB0b25zL2hlY3RhcmUpIGFuZCByZWR1Y3Rpb25zIGluIHdhdGVyIGFuZCBlbmVyZ3kgY29uc3VtcHRpb24uIFJlZ3Jlc3Npb24gYW5hbHlzaXMgaGlnaGxpZ2h0ZWQgc3Ryb25nIHBvc2l0aXZlIGVmZmVjdHMgb24geWllbGQgYW5kIGVmZmljaWVuY3ksIHdoaWxlIFNFTSBjb25maXJtZWQgcmVkdWNlZCBlbnZpcm9ubWVudGFsIGltcGFjdCBhbmQgb3BlcmF0aW9uYWwgY29zdHMuIFRoZSBzdHVkeSBjb25jbHVkZXMgdGhhdCBzb2xhci1wb3dlcmVkIGlycmlnYXRpb24gZW5oYW5jZXMgcHJvZHVjdGl2aXR5LCBjb25zZXJ2ZXMgcmVzb3VyY2VzLCBhbmQgcHJvbW90ZXMgc3VzdGFpbmFiaWxpdHkuIFBvbGljeW1ha2VycyBzaG91bGQgcHJvdmlkZSBmaW5hbmNpYWwgaW5jZW50aXZlcywgaW52ZXN0IGluIHJlbmV3YWJsZSBpbmZyYXN0cnVjdHVyZSwgYW5kIGltcGxlbWVudCB0cmFpbmluZyBwcm9ncmFtcyB0byBzdXBwb3J0IGFkb3B0aW9uLiBDb2xsYWJvcmF0aXZlIGVmZm9ydHMgYXJlIGVzc2VudGlhbCBmb3Igc3VzdGFpbmFibGUgYWdyaWN1bHR1cmUgaW4gYXJpZCByZWdpb25zLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJpc3N1ZWREYXRlIjoiMjAyNS0wMy0wNSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkFncmljdWx0dXJlIiwiaXNzdWUiOjUsInZvbHVtZSI6MTUsImltcGFjdEZhY3RvciI6IjUuMjYifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjo1NTQsImxhc3RQYWdlIjo1NTR9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiTXVsdGlkaXNjaXBsaW5hcnkgRGlnaXRhbCBQdWJsaXNoaW5nIEluc3RpdHV0ZSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NDgsImNpdGVkQnlDb3VudCI6Mn19LCJxdW90ZSI6IjEuIEludHJvZHVjdGlvbiBUaGUgYWdyaWN1bHR1cmFsIHNlY3RvciwgYSBjb3JuZXJzdG9uZSBvZiBnbG9iYWwgZm9vZCBzZWN1cml0eSBhbmQgZWNvbm9taWMgZGV2ZWxvcG1lbnQsIGZhY2VzIG1vdW50aW5nIGNoYWxsZW5nZXMgaW4gdGhlIDIxc3QgY2VudHVyeS4gQ2xpbWF0ZSBjaGFuZ2UsIHJhcGlkIHBvcHVsYXRpb24gZ3Jvd3RoLCBhbmQgZHdpbmRsaW5nIG5hdHVyYWwgcmVzb3VyY2VzIGhhdmUgaW50ZW5zaWZpZWQgdGhlIG5lZWQgZm9yIHN1c3RhaW5hYmxlIGFncmljdWx0dXJhbCBwcmFjdGljZXMuIEFtb25nIHRoZXNlIGNoYWxsZW5nZXMsIHdhdGVyIHNjYXJjaXR5IGFuZCBlbmVyZ3kgaW5lZmZpY2llbmN5IHN0YW5kIG91dCBhcyBjcml0aWNhbCBiYXJyaWVycyB0byBwcm9kdWN0aXZpdHkgYW5kIHN1c3RhaW5hYmlsaXR5LiBHbG9iYWxseSwgYWdyaWN1bHR1cmUgYWNjb3VudHMgZm9yIGFwcHJveGltYXRlbHkgNzAlIG9mIGZyZXNod2F0ZXIgd2l0aGRyYXdhbHMgYW5kIGlzIG9uZSBvZiB0aGUgbGFyZ2VzdCBjb25zdW1lcnMgb2YgZW5lcmd5IGluIG1hbnkgcmVnaW9ucyBbIDEsIDJdLiBUcmFkaXRpb25hbCBpcnJpZ2F0aW9uIHByYWN0aWNlcywgb2Z0ZW4gaW5lZmZpY2llbnQgYW5kIHJlc291cmNlLWludGVuc2l2ZSwgZXhhY2VyYmF0ZSB0aGVzZSBpc3N1ZXMsIGxlYWRpbmcgdG8gd2F0ZXIgd2FzdGFnZSwgZW5lcmd5IG92ZXJ1c2UsIGFuZCByZWR1Y2VkIGNyb3AgeWllbGRzLiJ9XQ%3D%3D;

- Energy optimization: Efficient for flat terrains, though higher than drip due to pressure needs (Daraz et al., 2025)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlVtYXIiLCJmYW1pbHkiOiJEYXJheiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiLFoHRlZmFuIiwiZmFtaWx5IjoiQm9qbmVjIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllvdW5hcyIsImZhbWlseSI6IktoYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMDc3LTA0NzIiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzOTAvYWdyaWN1bHR1cmUxNTA1MDU1NCIsIm9wZW5hbGV4IjoiVzQ0MDgyMTk4MTEiLCJleGEiOiJodHRwczovL3d3dy5tZHBpLmNvbS8yMDc3LTA0NzIvMTUvNS81NTQifSwidGl0bGUiOiJFbmVyZ3ktRWZmaWNpZW50IFNtYXJ0IElycmlnYXRpb24gVGVjaG5vbG9naWVzOiBBIFBhdGh3YXkgdG8gV2F0ZXIgYW5kIEVuZXJneSBTdXN0YWluYWJpbGl0eSBpbiBBZ3JpY3VsdHVyZSIsImFic3RyYWN0IjoiVGhlIGFncmljdWx0dXJhbCBzZWN0b3IgZmFjZXMgY2hhbGxlbmdlcyBzdWNoIGFzIHdhdGVyIHNjYXJjaXR5LCBlbmVyZ3kgaW5lZmZpY2llbmN5LCBhbmQgZGVjbGluaW5nIHByb2R1Y3Rpdml0eSwgcGFydGljdWxhcmx5IGluIGFyaWQgcmVnaW9ucy4gVHJhZGl0aW9uYWwgaXJyaWdhdGlvbiBtZXRob2RzIGNvbnRyaWJ1dGUgdG8gcmVzb3VyY2UgZGVwbGV0aW9uIGFuZCBlbnZpcm9ubWVudGFsIGltcGFjdHMuIFNvbGFyLXBvd2VyZWQgc21hcnQgaXJyaWdhdGlvbiBzeXN0ZW1zIGludGVncmF0ZSBwcmVjaXNpb24gaXJyaWdhdGlvbiB3aXRoIHJlbmV3YWJsZSBlbmVyZ3ksIGltcHJvdmluZyB3YXRlciB1c2UgYW5kIHByb2R1Y3Rpdml0eS4gSW4gUGFraXN0YW4sIHdoZXJlIGFncmljdWx0dXJlIGNvbnRyaWJ1dGVzIDE5JSBvZiBncm9zcyBkb21lc3RpYyBwcm9kdWN0IGFuZCBlbXBsb3lzIDQwJSBvZiB0aGUgd29ya2ZvcmNlLCB0aGVzZSBjaGFsbGVuZ2VzIGFyZSBzZXZlcmUsIGVzcGVjaWFsbHkgaW4gd2F0ZXItc2NhcmNlIGFyZWFzIGxpa2UgdGhlIENob2xpc3RhbiBEZXNlcnQuIFRoaXMgc3R1ZHkgZXhhbWluZXMgdGhlIGltcGFjdCBvZiBzb2xhci1wb3dlcmVkIHNtYXJ0IGlycmlnYXRpb24gb24gYWdyaWN1bHR1cmFsIHByb2R1Y3Rpdml0eSwgd2F0ZXIgY29uc2VydmF0aW9uLCBhbmQgZW5lcmd5IGVmZmljaWVuY3kgaW4gdGhlIENob2xpc3RhbiBEZXNlcnQuIFVzaW5nIGEgcXVhbnRpdGF0aXZlIGNyb3NzLXNlY3Rpb25hbCBkZXNpZ24sIGRhdGEgd2VyZSBjb2xsZWN0ZWQgZnJvbSAzODQgZmFybWVycyB2aWEgc3RydWN0dXJlZCBxdWVzdGlvbm5haXJlcy4gU3RhdGlzdGljYWwgYW5hbHlzZXMsIGluY2x1ZGluZyBtdWx0aXBsZSBsaW5lYXIgcmVncmVzc2lvbiwgcGFpcmVkIHNhbXBsZSB0LXRlc3RzLCBhbmQgU3RydWN0dXJhbCBFcXVhdGlvbiBNb2RlbGluZyAoU0VNKSwgd2VyZSBjb25kdWN0ZWQuIEZpbmRpbmdzIHNob3cgc2lnbmlmaWNhbnQgaW1wcm92ZW1lbnRzIGluIGNyb3AgeWllbGQgKGZyb20gMy4wIHRvIDQuOCB0b25zL2hlY3RhcmUpIGFuZCByZWR1Y3Rpb25zIGluIHdhdGVyIGFuZCBlbmVyZ3kgY29uc3VtcHRpb24uIFJlZ3Jlc3Npb24gYW5hbHlzaXMgaGlnaGxpZ2h0ZWQgc3Ryb25nIHBvc2l0aXZlIGVmZmVjdHMgb24geWllbGQgYW5kIGVmZmljaWVuY3ksIHdoaWxlIFNFTSBjb25maXJtZWQgcmVkdWNlZCBlbnZpcm9ubWVudGFsIGltcGFjdCBhbmQgb3BlcmF0aW9uYWwgY29zdHMuIFRoZSBzdHVkeSBjb25jbHVkZXMgdGhhdCBzb2xhci1wb3dlcmVkIGlycmlnYXRpb24gZW5oYW5jZXMgcHJvZHVjdGl2aXR5LCBjb25zZXJ2ZXMgcmVzb3VyY2VzLCBhbmQgcHJvbW90ZXMgc3VzdGFpbmFiaWxpdHkuIFBvbGljeW1ha2VycyBzaG91bGQgcHJvdmlkZSBmaW5hbmNpYWwgaW5jZW50aXZlcywgaW52ZXN0IGluIHJlbmV3YWJsZSBpbmZyYXN0cnVjdHVyZSwgYW5kIGltcGxlbWVudCB0cmFpbmluZyBwcm9ncmFtcyB0byBzdXBwb3J0IGFkb3B0aW9uLiBDb2xsYWJvcmF0aXZlIGVmZm9ydHMgYXJlIGVzc2VudGlhbCBmb3Igc3VzdGFpbmFibGUgYWdyaWN1bHR1cmUgaW4gYXJpZCByZWdpb25zLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJpc3N1ZWREYXRlIjoiMjAyNS0wMy0wNSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkFncmljdWx0dXJlIiwiaXNzdWUiOjUsInZvbHVtZSI6MTUsImltcGFjdEZhY3RvciI6IjUuMjYifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjo1NTQsImxhc3RQYWdlIjo1NTR9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiTXVsdGlkaXNjaXBsaW5hcnkgRGlnaXRhbCBQdWJsaXNoaW5nIEluc3RpdHV0ZSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NDgsImNpdGVkQnlDb3VudCI6Mn19LCJxdW90ZSI6IjEuIEludHJvZHVjdGlvbiBUaGUgYWdyaWN1bHR1cmFsIHNlY3RvciwgYSBjb3JuZXJzdG9uZSBvZiBnbG9iYWwgZm9vZCBzZWN1cml0eSBhbmQgZWNvbm9taWMgZGV2ZWxvcG1lbnQsIGZhY2VzIG1vdW50aW5nIGNoYWxsZW5nZXMgaW4gdGhlIDIxc3QgY2VudHVyeS4gQ2xpbWF0ZSBjaGFuZ2UsIHJhcGlkIHBvcHVsYXRpb24gZ3Jvd3RoLCBhbmQgZHdpbmRsaW5nIG5hdHVyYWwgcmVzb3VyY2VzIGhhdmUgaW50ZW5zaWZpZWQgdGhlIG5lZWQgZm9yIHN1c3RhaW5hYmxlIGFncmljdWx0dXJhbCBwcmFjdGljZXMuIEFtb25nIHRoZXNlIGNoYWxsZW5nZXMsIHdhdGVyIHNjYXJjaXR5IGFuZCBlbmVyZ3kgaW5lZmZpY2llbmN5IHN0YW5kIG91dCBhcyBjcml0aWNhbCBiYXJyaWVycyB0byBwcm9kdWN0aXZpdHkgYW5kIHN1c3RhaW5hYmlsaXR5LiBHbG9iYWxseSwgYWdyaWN1bHR1cmUgYWNjb3VudHMgZm9yIGFwcHJveGltYXRlbHkgNzAlIG9mIGZyZXNod2F0ZXIgd2l0aGRyYXdhbHMgYW5kIGlzIG9uZSBvZiB0aGUgbGFyZ2VzdCBjb25zdW1lcnMgb2YgZW5lcmd5IGluIG1hbnkgcmVnaW9ucyBbIDEsIDJdLiBUcmFkaXRpb25hbCBpcnJpZ2F0aW9uIHByYWN0aWNlcywgb2Z0ZW4gaW5lZmZpY2llbnQgYW5kIHJlc291cmNlLWludGVuc2l2ZSwgZXhhY2VyYmF0ZSB0aGVzZSBpc3N1ZXMsIGxlYWRpbmcgdG8gd2F0ZXIgd2FzdGFnZSwgZW5lcmd5IG92ZXJ1c2UsIGFuZCByZWR1Y2VkIGNyb3AgeWllbGRzLiJ9XQ%3D%3D;

- Yield enhancements: Even coverage improves uniformity and reduces weed pressure (Jumintono et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikp1bWludG9ubyIsImZhbWlseSI6Ikp1bWludG9ubyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJOdXJuYXdhdHkiLCJmYW1pbHkiOiJOdXJuYXdhdHkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTnVncm9obyBUcmlzbnUiLCJmYW1pbHkiOiJCcmF0YSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJEaW5pIiwiZmFtaWx5IjoiUmFobWlhdGkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSGFkaSIsImZhbWlseSI6IlN1c2lsbyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBLiBBLiIsImZhbWlseSI6Ikh1c3NlbiJ9fV0sImlkcyI6eyJpc3NuIjpbIjE0MjktNzQyNiIsIjIwODMtNDUzNSJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMjQ0MjUvandsZC4yMDI0LjE1MTc5OCIsIm9wZW5hbGV4IjoiVzQ0MDU0NzcxMDAifSwidGl0bGUiOiJDb21wYXJhdGl2ZSBhbmFseXNpcyBvZiBwcmVjaXNpb24sIGRlZmljaXQsIGFuZCBkcmlwIGlycmlnYXRpb24gZm9yIHdhdGVyIGVmZmljaWVuY3kgaW4gSXJhcSIsImFic3RyYWN0IjoiU3VzdGFpbmFibGUgaXJyaWdhdGlvbiBwcmFjdGljZXMgYXJlIGNyaXRpY2FsIGZvciBhZGRyZXNzaW5nIHRoZSBjaGFsbGVuZ2VzIG9mIHdhdGVyIHNjYXJjaXR5IGFuZCBmb29kIGluc2VjdXJpdHkgaW4gdGhlIGZhY2Ugb2YgY2xpbWF0ZSBjaGFuZ2UgYW5kIGdyb3dpbmcgZ2xvYmFsIGZvb2QgZGVtYW5kLiBUaGlzIHN0dWR5IGludmVzdGlnYXRlZCB0aGUgcG90ZW50aWFsIG9mIHByZWNpc2lvbiBpcnJpZ2F0aW9uIChQSSksIGRlZmljaXQgaXJyaWdhdGlvbiAoREkpLCBhbmQgZHJpcCBpcnJpZ2F0aW9uIChEUkkpIHRlY2huaXF1ZXMgdG8gZW5oYW5jZSBjcm9wIHByb2R1Y3Rpdml0eSBhbmQgd2F0ZXIgdXNlIGVmZmljaWVuY3kgKFdVRSkgY29tcGFyZWQgdG8gY29udmVudGlvbmFsIGZsb29kIGlycmlnYXRpb24gKENGSSkgaW4gdGhyZWUgZGlzdGluY3QgYWdyb2Vjb2xvZ2ljYWwgem9uZXMgb2YgSXJhcS4gRmllbGQgZXhwZXJpbWVudHMgd2VyZSBjb25kdWN0ZWQgdXNpbmcgYSByYW5kb21pc2VkIGNvbXBsZXRlIGJsb2NrIGRlc2lnbiB3aXRoIHdoZWF0LCBtYWl6ZSwgYW5kIHJpY2UgY3JvcHMuIFRoZSByZXN1bHRzIGRlbW9uc3RyYXRlZCB0aGF0IFBJIHNpZ25pZmljYW50bHkgaW5jcmVhc2VkIGNyb3AgZ3Jvd3RoIHBhcmFtZXRlcnMsIGdyYWluIHlpZWxkLCBhbmQgV1VFIGFjcm9zcyBhbGwgem9uZXMsIHdpdGggeWllbGQgaW1wcm92ZW1lbnRzIG9mIDMz4oCTMzglIGFuZCBXVUUgaW5jcmVhc2VzIG9mIDQ24oCTNTElIGluIGNvbnRyYXN0IHdpdGggQ0ZJLiBUaGUgREkgYW5kIERSSSB0cmVhdG1lbnRzIGFsc28gb3V0cGVyZm9ybWVkIENGSSwgYWxiZWl0IHRvIGEgbGVzc2VyIGV4dGVudC4gUmVtb3RlIHNlbnNpbmctZGVyaXZlZCB2ZWdldGF0aW9uIGluZGljZXMgc3Ryb25nbHkgY29ycmVsYXRlZCB3aXRoIGNyb3AgZ3Jvd3RoIHBhcmFtZXRlcnMgYW5kIHlpZWxkLCB3aGlsZSBoeWRyb2xvZ2ljYWwgbW9kZWxsaW5nIHJldmVhbGVkIHJlZHVjZWQgZXZhcG90cmFuc3BpcmF0aW9uIGFuZCBzdXJmYWNlIHJ1bm9mZiB1bmRlciB0aGUgUEkgdHJlYXRtZW50LiBUaGUgc3VzdGFpbmFibGUgaXJyaWdhdGlvbiBwcmFjdGljZXMgcmVzdWx0ZWQgaW4gc3Vic3RhbnRpYWwgd2F0ZXIgc2F2aW5ncyBvZiAyMOKAkzMwJSBjb21wYXJlZCB0byBDRkkuIFRoZXNlIGZpbmRpbmdzIGhpZ2hsaWdodCB0aGUgaW1wb3J0YW5jZSBvZiBhZG9wdGluZyBlZmZpY2llbnQgaXJyaWdhdGlvbiB0ZWNobmlxdWVzLCBhbG9uZyB3aXRoIGEgaG9saXN0aWMgYXBwcm9hY2ggZW5jb21wYXNzaW5nIHRlY2hub2xvZ2ljYWwgaW5ub3ZhdGlvbnMsIGNhcGFjaXR5IGJ1aWxkaW5nLCBhbmQgc3Rha2Vob2xkZXIgZW5nYWdlbWVudCwgdG8gcHJvbW90ZSB3YXRlci0gZWZmaWNpZW50IGFncmljdWx0dXJlIGFuZCBlbnN1cmUgZm9vZCBzZWN1cml0eSBpbiB3YXRlci1zY2FyY2UgcmVnaW9ucy4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjQ0MjUvandsZC4yMDI0LjE1MTc5OCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjI0NDI1L2p3bGQuMjAyNC4xNTE3OTgiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMi0xNCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkpvdXJuYWwgb2YgV2F0ZXIgYW5kIExhbmQgRGV2ZWxvcG1lbnQiLCJpbXBhY3RGYWN0b3IiOiIxLjQ2In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTMwLCJsYXN0UGFnZSI6MTM2fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkRlIEdydXl0ZXIgT3BlbiJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MzIsImNpdGVkQnlDb3VudCI6MX19LCJxdW90ZSI6IkluIHRoaXMgY29udGV4dCwgdGhlIGFkb3B0aW9uIG9mICBzdXN0YWluYWJsZSBpcnJpZ2F0aW9uIHByYWN0aWNlcyBoYXMgZW1lcmdlZCBhcyBhIGNyaXRpY2FsIHN0cmF0ZWd5ICBmb3IgZW5oYW5jaW5nIGZvb2Qgc2VjdXJpdHkgd2hpbGUgcHJvbW90aW5nIHdhdGVyIGNvbnNlcnZhdGlvbiAgaW4gYWdyaWN1bHR1cmFsIGxhbmRzY2FwZXMuICBJcnJpZ2F0aW9uIHBsYXlzIGEgdml0YWwgcm9sZSBpbiBnbG9iYWwgZm9vZCBwcm9kdWN0aW9uLCB3aXRoICBpcnJpZ2F0ZWQgYWdyaWN1bHR1cmUgYWNjb3VudGluZyBmb3IgYXBwcm94aW1hdGVseSA0MCUgb2YgdGhlICBmb29kIHN1cHBseSAoV3JhY2hpZW4gZGUsIFNjaHVsdHogYW5kIEdvbGksIDIwMjE7IFNjaG1pdHQsIFJvc2EgIGFuZCBEYWlseSwgMjAyMikuIEhvd2V2ZXIsIHRyYWRpdGlvbmFsIGlycmlnYXRpb24gbWV0aG9kcywgc3VjaCBhcyAgZmxvb2QgYW5kIGZ1cnJvdyBpcnJpZ2F0aW9uLCBhcmUgb2Z0ZW4gaW5lZmZpY2llbnQsIGxlYWRpbmcgdG8gIHN1YnN0YW50aWFsIHdhdGVyIGxvc3NlcyB0aHJvdWdoIGV2YXBvcmF0aW9uLCBydW5vZmYsIGFuZCBkZWVwICBwZXJjb2xhdGlvbiAoQW1ib21zYSwgMjAyMCkuIE1vcmVvdmVyLCB0aGUgZXhjZXNzaXZlIGFwcGxpY2EtdGlvbiBvZiB3YXRlciBjYW4gcmVzdWx0IGluIHNvaWwgc2FsaW5pc2F0aW9uLCBudXRyaWVudCBsZWFjaGluZywgYW5kICBncm91bmR3YXRlciBkZXBsZXRpb24sIGZ1cnRoZXIgY29tcHJvbWlzaW5nIHRoZSBsb25nLXRlcm0gIHN1c3RhaW5hYmlsaXR5IG9mIGFncmljdWx0dXJhbCBzeXN0ZW1zIChZaW4gZXQgYWwuLCAyMDIyKS4gVG8gYWRkcmVzcyAgdGhlc2UgY2hhbGxlbmdlcywgcmVzZWFyY2hlcnMgYW5kIHByYWN0aXRpb25lcnMgaGF2ZSBiZWVuIGV4cGxvci1pbmcgaW5ub3ZhdGl2ZSBpcnJpZ2F0aW9uIHRlY2huaXF1ZXMgYW5kIHdhdGVyIG1hbmFnZW1lbnQgIHN0cmF0ZWdpZXMgdGhhdCBjYW4gb3B0aW1pc2Ugd2F0ZXIgdXNlIGVmZmljaWVuY3kgKFdVRSkgd2hpbGUgIG1haW50YWluaW5nIG9yIGVuaGFuY2luZyBjcm9wIHByb2R1Y3Rpdml0eS4gIFByZWNpc2lvbiBpcnJpZ2F0aW9uIChQSSkgb2ZmZXJzIGEgcHJvbWlzaW5nIHNvbHV0aW9uLCAgaW52b2x2aW5nIHRoZSB0YXJnZXRlZCBhcHBsaWNhdGlvbiBvZiB3YXRlciBzcGVjaWZpY2FsbHkgdG8gdGhlICBuZWVkcyBvZiBpbmRpdmlkdWFsIHBsYW50cyBvciBjcm9wIHpvbmVzIChaaW5rZXJuYWdlbCBldCBhbC4sIDIwMjApLiAgIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyMi44ODY3MzU5MTYxMzc2OTUsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzQ2MjIxOTIzODI4MTI1LCJoZWlnaHQiOjEuNDUwNTQxMDE5NDM5Njk3M319LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI0LjM1MTEyMzgwOTgxNDQ1MywibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTI3MTYwNjQ0NSwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI1LjcyNDk4NzAzMDAyOTI5NywibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTY1MzA3NjE3MiwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI3LjA5ODg1MDI1MDI0NDE0LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjE2LjQxODE2MTM5MjIxMTkxNCwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI4LjQ3MjcwOTY1NTc2MTcyLCJsZWZ0IjoxMi4zODA4ODAzNTU4MzQ5NjEsIndpZHRoIjozNy4wMTIxODQxNDMwNjY0MDYsImhlaWdodCI6MS4zNjAwMTQ3OTYyNTcwMTl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyOS44NDY1NzI4NzU5NzY1NjIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQwNjQ4NjUxMTIzMDUsImhlaWdodCI6MS4zNjAwMTY3MDM2MDU2NTE5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzEuMjE0MDM2OTQxNTI4MzIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQ0ODU2MjYyMjA3MDMsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzIuNTg3OTAyMDY5MDkxOCwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDYwNTc4OTE4NDU3LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMzLjk2MTc2MTQ3NDYwOTM3NSwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDA0ODQ2MTkxNDA2MjUsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzUuMzM1NjI0Njk0ODI0MjIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzM5NzQ0NTY3ODcxMDk0LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjM2LjcwOTQ4NzkxNTAzOTA2LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjAwMTU5MDcyODc1OTc2NiwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC4wODMzNDczMjA1NTY2NCwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTY1MzA3NjE3MiwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOS40NTcyMTA1NDA3NzE0ODQsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQzMDQ4MDk1NzAzMTI1LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQwLjc0MDU0NzE4MDE3NTc4LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjM0NTQ1NTE2OTY3NzczNCwiaGVpZ2h0IjoxLjQ1MDU0MjgwNzU3OTA0MDV9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Mi4yMDQ5MzMxNjY1MDM5MDYsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMDAzMjUwMTIyMDcwMzEsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDMuNTc4Nzk2Mzg2NzE4NzUsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzM3OTIxMTQyNTc4MTI1LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQ0Ljg2MjEzMzAyNjEyMzA1LCJsZWZ0Ijo5LjA0NzYwMTY5OTgyOTEwMiwid2lkdGgiOjQwLjM0NTMxNDAyNTg3ODkwNiwiaGVpZ2h0IjoxLjQ1MDU0MjgwNzU3OTA0MDV9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ni4zMjY1MTkwMTI0NTExNywibGVmdCI6OS4wNDc2MDE2OTk4MjkxMDIsIndpZHRoIjoyNy45Njc0NDcyODA4ODM3OSwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ny43MDAzODIyMzI2NjYwMTYsImxlZnQiOjEyLjM4MDg3ODQ0ODQ4NjMyOCwid2lkdGgiOjM3LjAzNDc4NjIyNDM2NTIzNCwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0OS4wNzQyNDU0NTI4ODA4NiwibGVmdCI6OS4wNDc2MDE2OTk4MjkxMDIsIndpZHRoIjo0MC4zNDA2NDQ4MzY0MjU3OCwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1MC4zNTc1ODIwOTIyODUxNTYsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzQ0ODUyNDQ3NTA5NzY2LCJoZWlnaHQiOjEuNDUwNTQyODA3NTc5MDQwNX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEzMSJ9fV0%3D;

- Versatility: Suitable for various crops and soils, curbing erosion.

Applications and Implementation

Common in row crops, pastures, and grains across the U.S., Australia, and arid zones; pivots excel in large-scale farming with automation for variable-rate application (Daraz et al., 2025)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlVtYXIiLCJmYW1pbHkiOiJEYXJheiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiLFoHRlZmFuIiwiZmFtaWx5IjoiQm9qbmVjIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllvdW5hcyIsImZhbWlseSI6IktoYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMDc3LTA0NzIiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzOTAvYWdyaWN1bHR1cmUxNTA1MDU1NCIsIm9wZW5hbGV4IjoiVzQ0MDgyMTk4MTEiLCJleGEiOiJodHRwczovL3d3dy5tZHBpLmNvbS8yMDc3LTA0NzIvMTUvNS81NTQifSwidGl0bGUiOiJFbmVyZ3ktRWZmaWNpZW50IFNtYXJ0IElycmlnYXRpb24gVGVjaG5vbG9naWVzOiBBIFBhdGh3YXkgdG8gV2F0ZXIgYW5kIEVuZXJneSBTdXN0YWluYWJpbGl0eSBpbiBBZ3JpY3VsdHVyZSIsImFic3RyYWN0IjoiVGhlIGFncmljdWx0dXJhbCBzZWN0b3IgZmFjZXMgY2hhbGxlbmdlcyBzdWNoIGFzIHdhdGVyIHNjYXJjaXR5LCBlbmVyZ3kgaW5lZmZpY2llbmN5LCBhbmQgZGVjbGluaW5nIHByb2R1Y3Rpdml0eSwgcGFydGljdWxhcmx5IGluIGFyaWQgcmVnaW9ucy4gVHJhZGl0aW9uYWwgaXJyaWdhdGlvbiBtZXRob2RzIGNvbnRyaWJ1dGUgdG8gcmVzb3VyY2UgZGVwbGV0aW9uIGFuZCBlbnZpcm9ubWVudGFsIGltcGFjdHMuIFNvbGFyLXBvd2VyZWQgc21hcnQgaXJyaWdhdGlvbiBzeXN0ZW1zIGludGVncmF0ZSBwcmVjaXNpb24gaXJyaWdhdGlvbiB3aXRoIHJlbmV3YWJsZSBlbmVyZ3ksIGltcHJvdmluZyB3YXRlciB1c2UgYW5kIHByb2R1Y3Rpdml0eS4gSW4gUGFraXN0YW4sIHdoZXJlIGFncmljdWx0dXJlIGNvbnRyaWJ1dGVzIDE5JSBvZiBncm9zcyBkb21lc3RpYyBwcm9kdWN0IGFuZCBlbXBsb3lzIDQwJSBvZiB0aGUgd29ya2ZvcmNlLCB0aGVzZSBjaGFsbGVuZ2VzIGFyZSBzZXZlcmUsIGVzcGVjaWFsbHkgaW4gd2F0ZXItc2NhcmNlIGFyZWFzIGxpa2UgdGhlIENob2xpc3RhbiBEZXNlcnQuIFRoaXMgc3R1ZHkgZXhhbWluZXMgdGhlIGltcGFjdCBvZiBzb2xhci1wb3dlcmVkIHNtYXJ0IGlycmlnYXRpb24gb24gYWdyaWN1bHR1cmFsIHByb2R1Y3Rpdml0eSwgd2F0ZXIgY29uc2VydmF0aW9uLCBhbmQgZW5lcmd5IGVmZmljaWVuY3kgaW4gdGhlIENob2xpc3RhbiBEZXNlcnQuIFVzaW5nIGEgcXVhbnRpdGF0aXZlIGNyb3NzLXNlY3Rpb25hbCBkZXNpZ24sIGRhdGEgd2VyZSBjb2xsZWN0ZWQgZnJvbSAzODQgZmFybWVycyB2aWEgc3RydWN0dXJlZCBxdWVzdGlvbm5haXJlcy4gU3RhdGlzdGljYWwgYW5hbHlzZXMsIGluY2x1ZGluZyBtdWx0aXBsZSBsaW5lYXIgcmVncmVzc2lvbiwgcGFpcmVkIHNhbXBsZSB0LXRlc3RzLCBhbmQgU3RydWN0dXJhbCBFcXVhdGlvbiBNb2RlbGluZyAoU0VNKSwgd2VyZSBjb25kdWN0ZWQuIEZpbmRpbmdzIHNob3cgc2lnbmlmaWNhbnQgaW1wcm92ZW1lbnRzIGluIGNyb3AgeWllbGQgKGZyb20gMy4wIHRvIDQuOCB0b25zL2hlY3RhcmUpIGFuZCByZWR1Y3Rpb25zIGluIHdhdGVyIGFuZCBlbmVyZ3kgY29uc3VtcHRpb24uIFJlZ3Jlc3Npb24gYW5hbHlzaXMgaGlnaGxpZ2h0ZWQgc3Ryb25nIHBvc2l0aXZlIGVmZmVjdHMgb24geWllbGQgYW5kIGVmZmljaWVuY3ksIHdoaWxlIFNFTSBjb25maXJtZWQgcmVkdWNlZCBlbnZpcm9ubWVudGFsIGltcGFjdCBhbmQgb3BlcmF0aW9uYWwgY29zdHMuIFRoZSBzdHVkeSBjb25jbHVkZXMgdGhhdCBzb2xhci1wb3dlcmVkIGlycmlnYXRpb24gZW5oYW5jZXMgcHJvZHVjdGl2aXR5LCBjb25zZXJ2ZXMgcmVzb3VyY2VzLCBhbmQgcHJvbW90ZXMgc3VzdGFpbmFiaWxpdHkuIFBvbGljeW1ha2VycyBzaG91bGQgcHJvdmlkZSBmaW5hbmNpYWwgaW5jZW50aXZlcywgaW52ZXN0IGluIHJlbmV3YWJsZSBpbmZyYXN0cnVjdHVyZSwgYW5kIGltcGxlbWVudCB0cmFpbmluZyBwcm9ncmFtcyB0byBzdXBwb3J0IGFkb3B0aW9uLiBDb2xsYWJvcmF0aXZlIGVmZm9ydHMgYXJlIGVzc2VudGlhbCBmb3Igc3VzdGFpbmFibGUgYWdyaWN1bHR1cmUgaW4gYXJpZCByZWdpb25zLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJpc3N1ZWREYXRlIjoiMjAyNS0wMy0wNSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkFncmljdWx0dXJlIiwiaXNzdWUiOjUsInZvbHVtZSI6MTUsImltcGFjdEZhY3RvciI6IjUuMjYifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjo1NTQsImxhc3RQYWdlIjo1NTR9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiTXVsdGlkaXNjaXBsaW5hcnkgRGlnaXRhbCBQdWJsaXNoaW5nIEluc3RpdHV0ZSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NDgsImNpdGVkQnlDb3VudCI6Mn19LCJxdW90ZSI6IjEuIEludHJvZHVjdGlvbiBUaGUgYWdyaWN1bHR1cmFsIHNlY3RvciwgYSBjb3JuZXJzdG9uZSBvZiBnbG9iYWwgZm9vZCBzZWN1cml0eSBhbmQgZWNvbm9taWMgZGV2ZWxvcG1lbnQsIGZhY2VzIG1vdW50aW5nIGNoYWxsZW5nZXMgaW4gdGhlIDIxc3QgY2VudHVyeS4gQ2xpbWF0ZSBjaGFuZ2UsIHJhcGlkIHBvcHVsYXRpb24gZ3Jvd3RoLCBhbmQgZHdpbmRsaW5nIG5hdHVyYWwgcmVzb3VyY2VzIGhhdmUgaW50ZW5zaWZpZWQgdGhlIG5lZWQgZm9yIHN1c3RhaW5hYmxlIGFncmljdWx0dXJhbCBwcmFjdGljZXMuIEFtb25nIHRoZXNlIGNoYWxsZW5nZXMsIHdhdGVyIHNjYXJjaXR5IGFuZCBlbmVyZ3kgaW5lZmZpY2llbmN5IHN0YW5kIG91dCBhcyBjcml0aWNhbCBiYXJyaWVycyB0byBwcm9kdWN0aXZpdHkgYW5kIHN1c3RhaW5hYmlsaXR5LiBHbG9iYWxseSwgYWdyaWN1bHR1cmUgYWNjb3VudHMgZm9yIGFwcHJveGltYXRlbHkgNzAlIG9mIGZyZXNod2F0ZXIgd2l0aGRyYXdhbHMgYW5kIGlzIG9uZSBvZiB0aGUgbGFyZ2VzdCBjb25zdW1lcnMgb2YgZW5lcmd5IGluIG1hbnkgcmVnaW9ucyBbIDEsIDJdLiBUcmFkaXRpb25hbCBpcnJpZ2F0aW9uIHByYWN0aWNlcywgb2Z0ZW4gaW5lZmZpY2llbnQgYW5kIHJlc291cmNlLWludGVuc2l2ZSwgZXhhY2VyYmF0ZSB0aGVzZSBpc3N1ZXMsIGxlYWRpbmcgdG8gd2F0ZXIgd2FzdGFnZSwgZW5lcmd5IG92ZXJ1c2UsIGFuZCByZWR1Y2VkIGNyb3AgeWllbGRzLiJ9XQ%3D%3D.

### Subsurface Drip Irrigation

Subsurface drip irrigation buries drip lines 15-60 cm below soil surface, delivering water directly to root zones with efficiencies exceeding 90-95%, surpassing surface drip by eliminating evaporation and surface disruptions (Jumintono et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikp1bWludG9ubyIsImZhbWlseSI6Ikp1bWludG9ubyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJOdXJuYXdhdHkiLCJmYW1pbHkiOiJOdXJuYXdhdHkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTnVncm9obyBUcmlzbnUiLCJmYW1pbHkiOiJCcmF0YSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJEaW5pIiwiZmFtaWx5IjoiUmFobWlhdGkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSGFkaSIsImZhbWlseSI6IlN1c2lsbyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBLiBBLiIsImZhbWlseSI6Ikh1c3NlbiJ9fV0sImlkcyI6eyJpc3NuIjpbIjE0MjktNzQyNiIsIjIwODMtNDUzNSJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMjQ0MjUvandsZC4yMDI0LjE1MTc5OCIsIm9wZW5hbGV4IjoiVzQ0MDU0NzcxMDAifSwidGl0bGUiOiJDb21wYXJhdGl2ZSBhbmFseXNpcyBvZiBwcmVjaXNpb24sIGRlZmljaXQsIGFuZCBkcmlwIGlycmlnYXRpb24gZm9yIHdhdGVyIGVmZmljaWVuY3kgaW4gSXJhcSIsImFic3RyYWN0IjoiU3VzdGFpbmFibGUgaXJyaWdhdGlvbiBwcmFjdGljZXMgYXJlIGNyaXRpY2FsIGZvciBhZGRyZXNzaW5nIHRoZSBjaGFsbGVuZ2VzIG9mIHdhdGVyIHNjYXJjaXR5IGFuZCBmb29kIGluc2VjdXJpdHkgaW4gdGhlIGZhY2Ugb2YgY2xpbWF0ZSBjaGFuZ2UgYW5kIGdyb3dpbmcgZ2xvYmFsIGZvb2QgZGVtYW5kLiBUaGlzIHN0dWR5IGludmVzdGlnYXRlZCB0aGUgcG90ZW50aWFsIG9mIHByZWNpc2lvbiBpcnJpZ2F0aW9uIChQSSksIGRlZmljaXQgaXJyaWdhdGlvbiAoREkpLCBhbmQgZHJpcCBpcnJpZ2F0aW9uIChEUkkpIHRlY2huaXF1ZXMgdG8gZW5oYW5jZSBjcm9wIHByb2R1Y3Rpdml0eSBhbmQgd2F0ZXIgdXNlIGVmZmljaWVuY3kgKFdVRSkgY29tcGFyZWQgdG8gY29udmVudGlvbmFsIGZsb29kIGlycmlnYXRpb24gKENGSSkgaW4gdGhyZWUgZGlzdGluY3QgYWdyb2Vjb2xvZ2ljYWwgem9uZXMgb2YgSXJhcS4gRmllbGQgZXhwZXJpbWVudHMgd2VyZSBjb25kdWN0ZWQgdXNpbmcgYSByYW5kb21pc2VkIGNvbXBsZXRlIGJsb2NrIGRlc2lnbiB3aXRoIHdoZWF0LCBtYWl6ZSwgYW5kIHJpY2UgY3JvcHMuIFRoZSByZXN1bHRzIGRlbW9uc3RyYXRlZCB0aGF0IFBJIHNpZ25pZmljYW50bHkgaW5jcmVhc2VkIGNyb3AgZ3Jvd3RoIHBhcmFtZXRlcnMsIGdyYWluIHlpZWxkLCBhbmQgV1VFIGFjcm9zcyBhbGwgem9uZXMsIHdpdGggeWllbGQgaW1wcm92ZW1lbnRzIG9mIDMz4oCTMzglIGFuZCBXVUUgaW5jcmVhc2VzIG9mIDQ24oCTNTElIGluIGNvbnRyYXN0IHdpdGggQ0ZJLiBUaGUgREkgYW5kIERSSSB0cmVhdG1lbnRzIGFsc28gb3V0cGVyZm9ybWVkIENGSSwgYWxiZWl0IHRvIGEgbGVzc2VyIGV4dGVudC4gUmVtb3RlIHNlbnNpbmctZGVyaXZlZCB2ZWdldGF0aW9uIGluZGljZXMgc3Ryb25nbHkgY29ycmVsYXRlZCB3aXRoIGNyb3AgZ3Jvd3RoIHBhcmFtZXRlcnMgYW5kIHlpZWxkLCB3aGlsZSBoeWRyb2xvZ2ljYWwgbW9kZWxsaW5nIHJldmVhbGVkIHJlZHVjZWQgZXZhcG90cmFuc3BpcmF0aW9uIGFuZCBzdXJmYWNlIHJ1bm9mZiB1bmRlciB0aGUgUEkgdHJlYXRtZW50LiBUaGUgc3VzdGFpbmFibGUgaXJyaWdhdGlvbiBwcmFjdGljZXMgcmVzdWx0ZWQgaW4gc3Vic3RhbnRpYWwgd2F0ZXIgc2F2aW5ncyBvZiAyMOKAkzMwJSBjb21wYXJlZCB0byBDRkkuIFRoZXNlIGZpbmRpbmdzIGhpZ2hsaWdodCB0aGUgaW1wb3J0YW5jZSBvZiBhZG9wdGluZyBlZmZpY2llbnQgaXJyaWdhdGlvbiB0ZWNobmlxdWVzLCBhbG9uZyB3aXRoIGEgaG9saXN0aWMgYXBwcm9hY2ggZW5jb21wYXNzaW5nIHRlY2hub2xvZ2ljYWwgaW5ub3ZhdGlvbnMsIGNhcGFjaXR5IGJ1aWxkaW5nLCBhbmQgc3Rha2Vob2xkZXIgZW5nYWdlbWVudCwgdG8gcHJvbW90ZSB3YXRlci0gZWZmaWNpZW50IGFncmljdWx0dXJlIGFuZCBlbnN1cmUgZm9vZCBzZWN1cml0eSBpbiB3YXRlci1zY2FyY2UgcmVnaW9ucy4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjQ0MjUvandsZC4yMDI0LjE1MTc5OCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjI0NDI1L2p3bGQuMjAyNC4xNTE3OTgiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMi0xNCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkpvdXJuYWwgb2YgV2F0ZXIgYW5kIExhbmQgRGV2ZWxvcG1lbnQiLCJpbXBhY3RGYWN0b3IiOiIxLjQ2In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTMwLCJsYXN0UGFnZSI6MTM2fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkRlIEdydXl0ZXIgT3BlbiJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MzIsImNpdGVkQnlDb3VudCI6MX19LCJxdW90ZSI6IkluIHRoaXMgY29udGV4dCwgdGhlIGFkb3B0aW9uIG9mICBzdXN0YWluYWJsZSBpcnJpZ2F0aW9uIHByYWN0aWNlcyBoYXMgZW1lcmdlZCBhcyBhIGNyaXRpY2FsIHN0cmF0ZWd5ICBmb3IgZW5oYW5jaW5nIGZvb2Qgc2VjdXJpdHkgd2hpbGUgcHJvbW90aW5nIHdhdGVyIGNvbnNlcnZhdGlvbiAgaW4gYWdyaWN1bHR1cmFsIGxhbmRzY2FwZXMuICBJcnJpZ2F0aW9uIHBsYXlzIGEgdml0YWwgcm9sZSBpbiBnbG9iYWwgZm9vZCBwcm9kdWN0aW9uLCB3aXRoICBpcnJpZ2F0ZWQgYWdyaWN1bHR1cmUgYWNjb3VudGluZyBmb3IgYXBwcm94aW1hdGVseSA0MCUgb2YgdGhlICBmb29kIHN1cHBseSAoV3JhY2hpZW4gZGUsIFNjaHVsdHogYW5kIEdvbGksIDIwMjE7IFNjaG1pdHQsIFJvc2EgIGFuZCBEYWlseSwgMjAyMikuIEhvd2V2ZXIsIHRyYWRpdGlvbmFsIGlycmlnYXRpb24gbWV0aG9kcywgc3VjaCBhcyAgZmxvb2QgYW5kIGZ1cnJvdyBpcnJpZ2F0aW9uLCBhcmUgb2Z0ZW4gaW5lZmZpY2llbnQsIGxlYWRpbmcgdG8gIHN1YnN0YW50aWFsIHdhdGVyIGxvc3NlcyB0aHJvdWdoIGV2YXBvcmF0aW9uLCBydW5vZmYsIGFuZCBkZWVwICBwZXJjb2xhdGlvbiAoQW1ib21zYSwgMjAyMCkuIE1vcmVvdmVyLCB0aGUgZXhjZXNzaXZlIGFwcGxpY2EtdGlvbiBvZiB3YXRlciBjYW4gcmVzdWx0IGluIHNvaWwgc2FsaW5pc2F0aW9uLCBudXRyaWVudCBsZWFjaGluZywgYW5kICBncm91bmR3YXRlciBkZXBsZXRpb24sIGZ1cnRoZXIgY29tcHJvbWlzaW5nIHRoZSBsb25nLXRlcm0gIHN1c3RhaW5hYmlsaXR5IG9mIGFncmljdWx0dXJhbCBzeXN0ZW1zIChZaW4gZXQgYWwuLCAyMDIyKS4gVG8gYWRkcmVzcyAgdGhlc2UgY2hhbGxlbmdlcywgcmVzZWFyY2hlcnMgYW5kIHByYWN0aXRpb25lcnMgaGF2ZSBiZWVuIGV4cGxvci1pbmcgaW5ub3ZhdGl2ZSBpcnJpZ2F0aW9uIHRlY2huaXF1ZXMgYW5kIHdhdGVyIG1hbmFnZW1lbnQgIHN0cmF0ZWdpZXMgdGhhdCBjYW4gb3B0aW1pc2Ugd2F0ZXIgdXNlIGVmZmljaWVuY3kgKFdVRSkgd2hpbGUgIG1haW50YWluaW5nIG9yIGVuaGFuY2luZyBjcm9wIHByb2R1Y3Rpdml0eS4gIFByZWNpc2lvbiBpcnJpZ2F0aW9uIChQSSkgb2ZmZXJzIGEgcHJvbWlzaW5nIHNvbHV0aW9uLCAgaW52b2x2aW5nIHRoZSB0YXJnZXRlZCBhcHBsaWNhdGlvbiBvZiB3YXRlciBzcGVjaWZpY2FsbHkgdG8gdGhlICBuZWVkcyBvZiBpbmRpdmlkdWFsIHBsYW50cyBvciBjcm9wIHpvbmVzIChaaW5rZXJuYWdlbCBldCBhbC4sIDIwMjApLiAgIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyMi44ODY3MzU5MTYxMzc2OTUsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzQ2MjIxOTIzODI4MTI1LCJoZWlnaHQiOjEuNDUwNTQxMDE5NDM5Njk3M319LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI0LjM1MTEyMzgwOTgxNDQ1MywibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTI3MTYwNjQ0NSwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI1LjcyNDk4NzAzMDAyOTI5NywibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTY1MzA3NjE3MiwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI3LjA5ODg1MDI1MDI0NDE0LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjE2LjQxODE2MTM5MjIxMTkxNCwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI4LjQ3MjcwOTY1NTc2MTcyLCJsZWZ0IjoxMi4zODA4ODAzNTU4MzQ5NjEsIndpZHRoIjozNy4wMTIxODQxNDMwNjY0MDYsImhlaWdodCI6MS4zNjAwMTQ3OTYyNTcwMTl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyOS44NDY1NzI4NzU5NzY1NjIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQwNjQ4NjUxMTIzMDUsImhlaWdodCI6MS4zNjAwMTY3MDM2MDU2NTE5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzEuMjE0MDM2OTQxNTI4MzIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQ0ODU2MjYyMjA3MDMsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzIuNTg3OTAyMDY5MDkxOCwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDYwNTc4OTE4NDU3LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMzLjk2MTc2MTQ3NDYwOTM3NSwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDA0ODQ2MTkxNDA2MjUsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzUuMzM1NjI0Njk0ODI0MjIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzM5NzQ0NTY3ODcxMDk0LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjM2LjcwOTQ4NzkxNTAzOTA2LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjAwMTU5MDcyODc1OTc2NiwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC4wODMzNDczMjA1NTY2NCwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTY1MzA3NjE3MiwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOS40NTcyMTA1NDA3NzE0ODQsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQzMDQ4MDk1NzAzMTI1LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQwLjc0MDU0NzE4MDE3NTc4LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjM0NTQ1NTE2OTY3NzczNCwiaGVpZ2h0IjoxLjQ1MDU0MjgwNzU3OTA0MDV9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Mi4yMDQ5MzMxNjY1MDM5MDYsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMDAzMjUwMTIyMDcwMzEsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDMuNTc4Nzk2Mzg2NzE4NzUsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzM3OTIxMTQyNTc4MTI1LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQ0Ljg2MjEzMzAyNjEyMzA1LCJsZWZ0Ijo5LjA0NzYwMTY5OTgyOTEwMiwid2lkdGgiOjQwLjM0NTMxNDAyNTg3ODkwNiwiaGVpZ2h0IjoxLjQ1MDU0MjgwNzU3OTA0MDV9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ni4zMjY1MTkwMTI0NTExNywibGVmdCI6OS4wNDc2MDE2OTk4MjkxMDIsIndpZHRoIjoyNy45Njc0NDcyODA4ODM3OSwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ny43MDAzODIyMzI2NjYwMTYsImxlZnQiOjEyLjM4MDg3ODQ0ODQ4NjMyOCwid2lkdGgiOjM3LjAzNDc4NjIyNDM2NTIzNCwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0OS4wNzQyNDU0NTI4ODA4NiwibGVmdCI6OS4wNDc2MDE2OTk4MjkxMDIsIndpZHRoIjo0MC4zNDA2NDQ4MzY0MjU3OCwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1MC4zNTc1ODIwOTIyODUxNTYsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzQ0ODUyNDQ3NTA5NzY2LCJoZWlnaHQiOjEuNDUwNTQyODA3NTc5MDQwNX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEzMSJ9fV0%3D. This method addresses precision needs in water-scarce regions, optimizing WUE while combating issues like salinization from traditional over-application (Jumintono et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikp1bWludG9ubyIsImZhbWlseSI6Ikp1bWludG9ubyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJOdXJuYXdhdHkiLCJmYW1pbHkiOiJOdXJuYXdhdHkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTnVncm9obyBUcmlzbnUiLCJmYW1pbHkiOiJCcmF0YSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJEaW5pIiwiZmFtaWx5IjoiUmFobWlhdGkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSGFkaSIsImZhbWlseSI6IlN1c2lsbyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBLiBBLiIsImZhbWlseSI6Ikh1c3NlbiJ9fV0sImlkcyI6eyJpc3NuIjpbIjE0MjktNzQyNiIsIjIwODMtNDUzNSJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMjQ0MjUvandsZC4yMDI0LjE1MTc5OCIsIm9wZW5hbGV4IjoiVzQ0MDU0NzcxMDAifSwidGl0bGUiOiJDb21wYXJhdGl2ZSBhbmFseXNpcyBvZiBwcmVjaXNpb24sIGRlZmljaXQsIGFuZCBkcmlwIGlycmlnYXRpb24gZm9yIHdhdGVyIGVmZmljaWVuY3kgaW4gSXJhcSIsImFic3RyYWN0IjoiU3VzdGFpbmFibGUgaXJyaWdhdGlvbiBwcmFjdGljZXMgYXJlIGNyaXRpY2FsIGZvciBhZGRyZXNzaW5nIHRoZSBjaGFsbGVuZ2VzIG9mIHdhdGVyIHNjYXJjaXR5IGFuZCBmb29kIGluc2VjdXJpdHkgaW4gdGhlIGZhY2Ugb2YgY2xpbWF0ZSBjaGFuZ2UgYW5kIGdyb3dpbmcgZ2xvYmFsIGZvb2QgZGVtYW5kLiBUaGlzIHN0dWR5IGludmVzdGlnYXRlZCB0aGUgcG90ZW50aWFsIG9mIHByZWNpc2lvbiBpcnJpZ2F0aW9uIChQSSksIGRlZmljaXQgaXJyaWdhdGlvbiAoREkpLCBhbmQgZHJpcCBpcnJpZ2F0aW9uIChEUkkpIHRlY2huaXF1ZXMgdG8gZW5oYW5jZSBjcm9wIHByb2R1Y3Rpdml0eSBhbmQgd2F0ZXIgdXNlIGVmZmljaWVuY3kgKFdVRSkgY29tcGFyZWQgdG8gY29udmVudGlvbmFsIGZsb29kIGlycmlnYXRpb24gKENGSSkgaW4gdGhyZWUgZGlzdGluY3QgYWdyb2Vjb2xvZ2ljYWwgem9uZXMgb2YgSXJhcS4gRmllbGQgZXhwZXJpbWVudHMgd2VyZSBjb25kdWN0ZWQgdXNpbmcgYSByYW5kb21pc2VkIGNvbXBsZXRlIGJsb2NrIGRlc2lnbiB3aXRoIHdoZWF0LCBtYWl6ZSwgYW5kIHJpY2UgY3JvcHMuIFRoZSByZXN1bHRzIGRlbW9uc3RyYXRlZCB0aGF0IFBJIHNpZ25pZmljYW50bHkgaW5jcmVhc2VkIGNyb3AgZ3Jvd3RoIHBhcmFtZXRlcnMsIGdyYWluIHlpZWxkLCBhbmQgV1VFIGFjcm9zcyBhbGwgem9uZXMsIHdpdGggeWllbGQgaW1wcm92ZW1lbnRzIG9mIDMz4oCTMzglIGFuZCBXVUUgaW5jcmVhc2VzIG9mIDQ24oCTNTElIGluIGNvbnRyYXN0IHdpdGggQ0ZJLiBUaGUgREkgYW5kIERSSSB0cmVhdG1lbnRzIGFsc28gb3V0cGVyZm9ybWVkIENGSSwgYWxiZWl0IHRvIGEgbGVzc2VyIGV4dGVudC4gUmVtb3RlIHNlbnNpbmctZGVyaXZlZCB2ZWdldGF0aW9uIGluZGljZXMgc3Ryb25nbHkgY29ycmVsYXRlZCB3aXRoIGNyb3AgZ3Jvd3RoIHBhcmFtZXRlcnMgYW5kIHlpZWxkLCB3aGlsZSBoeWRyb2xvZ2ljYWwgbW9kZWxsaW5nIHJldmVhbGVkIHJlZHVjZWQgZXZhcG90cmFuc3BpcmF0aW9uIGFuZCBzdXJmYWNlIHJ1bm9mZiB1bmRlciB0aGUgUEkgdHJlYXRtZW50LiBUaGUgc3VzdGFpbmFibGUgaXJyaWdhdGlvbiBwcmFjdGljZXMgcmVzdWx0ZWQgaW4gc3Vic3RhbnRpYWwgd2F0ZXIgc2F2aW5ncyBvZiAyMOKAkzMwJSBjb21wYXJlZCB0byBDRkkuIFRoZXNlIGZpbmRpbmdzIGhpZ2hsaWdodCB0aGUgaW1wb3J0YW5jZSBvZiBhZG9wdGluZyBlZmZpY2llbnQgaXJyaWdhdGlvbiB0ZWNobmlxdWVzLCBhbG9uZyB3aXRoIGEgaG9saXN0aWMgYXBwcm9hY2ggZW5jb21wYXNzaW5nIHRlY2hub2xvZ2ljYWwgaW5ub3ZhdGlvbnMsIGNhcGFjaXR5IGJ1aWxkaW5nLCBhbmQgc3Rha2Vob2xkZXIgZW5nYWdlbWVudCwgdG8gcHJvbW90ZSB3YXRlci0gZWZmaWNpZW50IGFncmljdWx0dXJlIGFuZCBlbnN1cmUgZm9vZCBzZWN1cml0eSBpbiB3YXRlci1zY2FyY2UgcmVnaW9ucy4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjQ0MjUvandsZC4yMDI0LjE1MTc5OCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjI0NDI1L2p3bGQuMjAyNC4xNTE3OTgiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMi0xNCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkpvdXJuYWwgb2YgV2F0ZXIgYW5kIExhbmQgRGV2ZWxvcG1lbnQiLCJpbXBhY3RGYWN0b3IiOiIxLjQ2In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTMwLCJsYXN0UGFnZSI6MTM2fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkRlIEdydXl0ZXIgT3BlbiJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MzIsImNpdGVkQnlDb3VudCI6MX19LCJxdW90ZSI6IkluIHRoaXMgY29udGV4dCwgdGhlIGFkb3B0aW9uIG9mICBzdXN0YWluYWJsZSBpcnJpZ2F0aW9uIHByYWN0aWNlcyBoYXMgZW1lcmdlZCBhcyBhIGNyaXRpY2FsIHN0cmF0ZWd5ICBmb3IgZW5oYW5jaW5nIGZvb2Qgc2VjdXJpdHkgd2hpbGUgcHJvbW90aW5nIHdhdGVyIGNvbnNlcnZhdGlvbiAgaW4gYWdyaWN1bHR1cmFsIGxhbmRzY2FwZXMuICBJcnJpZ2F0aW9uIHBsYXlzIGEgdml0YWwgcm9sZSBpbiBnbG9iYWwgZm9vZCBwcm9kdWN0aW9uLCB3aXRoICBpcnJpZ2F0ZWQgYWdyaWN1bHR1cmUgYWNjb3VudGluZyBmb3IgYXBwcm94aW1hdGVseSA0MCUgb2YgdGhlICBmb29kIHN1cHBseSAoV3JhY2hpZW4gZGUsIFNjaHVsdHogYW5kIEdvbGksIDIwMjE7IFNjaG1pdHQsIFJvc2EgIGFuZCBEYWlseSwgMjAyMikuIEhvd2V2ZXIsIHRyYWRpdGlvbmFsIGlycmlnYXRpb24gbWV0aG9kcywgc3VjaCBhcyAgZmxvb2QgYW5kIGZ1cnJvdyBpcnJpZ2F0aW9uLCBhcmUgb2Z0ZW4gaW5lZmZpY2llbnQsIGxlYWRpbmcgdG8gIHN1YnN0YW50aWFsIHdhdGVyIGxvc3NlcyB0aHJvdWdoIGV2YXBvcmF0aW9uLCBydW5vZmYsIGFuZCBkZWVwICBwZXJjb2xhdGlvbiAoQW1ib21zYSwgMjAyMCkuIE1vcmVvdmVyLCB0aGUgZXhjZXNzaXZlIGFwcGxpY2EtdGlvbiBvZiB3YXRlciBjYW4gcmVzdWx0IGluIHNvaWwgc2FsaW5pc2F0aW9uLCBudXRyaWVudCBsZWFjaGluZywgYW5kICBncm91bmR3YXRlciBkZXBsZXRpb24sIGZ1cnRoZXIgY29tcHJvbWlzaW5nIHRoZSBsb25nLXRlcm0gIHN1c3RhaW5hYmlsaXR5IG9mIGFncmljdWx0dXJhbCBzeXN0ZW1zIChZaW4gZXQgYWwuLCAyMDIyKS4gVG8gYWRkcmVzcyAgdGhlc2UgY2hhbGxlbmdlcywgcmVzZWFyY2hlcnMgYW5kIHByYWN0aXRpb25lcnMgaGF2ZSBiZWVuIGV4cGxvci1pbmcgaW5ub3ZhdGl2ZSBpcnJpZ2F0aW9uIHRlY2huaXF1ZXMgYW5kIHdhdGVyIG1hbmFnZW1lbnQgIHN0cmF0ZWdpZXMgdGhhdCBjYW4gb3B0aW1pc2Ugd2F0ZXIgdXNlIGVmZmljaWVuY3kgKFdVRSkgd2hpbGUgIG1haW50YWluaW5nIG9yIGVuaGFuY2luZyBjcm9wIHByb2R1Y3Rpdml0eS4gIFByZWNpc2lvbiBpcnJpZ2F0aW9uIChQSSkgb2ZmZXJzIGEgcHJvbWlzaW5nIHNvbHV0aW9uLCAgaW52b2x2aW5nIHRoZSB0YXJnZXRlZCBhcHBsaWNhdGlvbiBvZiB3YXRlciBzcGVjaWZpY2FsbHkgdG8gdGhlICBuZWVkcyBvZiBpbmRpdmlkdWFsIHBsYW50cyBvciBjcm9wIHpvbmVzIChaaW5rZXJuYWdlbCBldCBhbC4sIDIwMjApLiAgIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyMi44ODY3MzU5MTYxMzc2OTUsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzQ2MjIxOTIzODI4MTI1LCJoZWlnaHQiOjEuNDUwNTQxMDE5NDM5Njk3M319LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI0LjM1MTEyMzgwOTgxNDQ1MywibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTI3MTYwNjQ0NSwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI1LjcyNDk4NzAzMDAyOTI5NywibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTY1MzA3NjE3MiwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI3LjA5ODg1MDI1MDI0NDE0LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjE2LjQxODE2MTM5MjIxMTkxNCwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI4LjQ3MjcwOTY1NTc2MTcyLCJsZWZ0IjoxMi4zODA4ODAzNTU4MzQ5NjEsIndpZHRoIjozNy4wMTIxODQxNDMwNjY0MDYsImhlaWdodCI6MS4zNjAwMTQ3OTYyNTcwMTl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyOS44NDY1NzI4NzU5NzY1NjIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQwNjQ4NjUxMTIzMDUsImhlaWdodCI6MS4zNjAwMTY3MDM2MDU2NTE5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzEuMjE0MDM2OTQxNTI4MzIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQ0ODU2MjYyMjA3MDMsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzIuNTg3OTAyMDY5MDkxOCwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDYwNTc4OTE4NDU3LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMzLjk2MTc2MTQ3NDYwOTM3NSwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDA0ODQ2MTkxNDA2MjUsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzUuMzM1NjI0Njk0ODI0MjIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzM5NzQ0NTY3ODcxMDk0LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjM2LjcwOTQ4NzkxNTAzOTA2LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjAwMTU5MDcyODc1OTc2NiwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC4wODMzNDczMjA1NTY2NCwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTY1MzA3NjE3MiwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOS40NTcyMTA1NDA3NzE0ODQsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQzMDQ4MDk1NzAzMTI1LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQwLjc0MDU0NzE4MDE3NTc4LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjM0NTQ1NTE2OTY3NzczNCwiaGVpZ2h0IjoxLjQ1MDU0MjgwNzU3OTA0MDV9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Mi4yMDQ5MzMxNjY1MDM5MDYsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMDAzMjUwMTIyMDcwMzEsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDMuNTc4Nzk2Mzg2NzE4NzUsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzM3OTIxMTQyNTc4MTI1LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQ0Ljg2MjEzMzAyNjEyMzA1LCJsZWZ0Ijo5LjA0NzYwMTY5OTgyOTEwMiwid2lkdGgiOjQwLjM0NTMxNDAyNTg3ODkwNiwiaGVpZ2h0IjoxLjQ1MDU0MjgwNzU3OTA0MDV9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ni4zMjY1MTkwMTI0NTExNywibGVmdCI6OS4wNDc2MDE2OTk4MjkxMDIsIndpZHRoIjoyNy45Njc0NDcyODA4ODM3OSwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ny43MDAzODIyMzI2NjYwMTYsImxlZnQiOjEyLjM4MDg3ODQ0ODQ4NjMyOCwid2lkdGgiOjM3LjAzNDc4NjIyNDM2NTIzNCwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0OS4wNzQyNDU0NTI4ODA4NiwibGVmdCI6OS4wNDc2MDE2OTk4MjkxMDIsIndpZHRoIjo0MC4zNDA2NDQ4MzY0MjU3OCwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1MC4zNTc1ODIwOTIyODUxNTYsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzQ0ODUyNDQ3NTA5NzY2LCJoZWlnaHQiOjEuNDUwNTQyODA3NTc5MDQwNX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEzMSJ9fV0%3D.

### Table 4: Precision Agriculture and Smart Technologies

| Component | Functionality | Key Benefit |
| --- | --- | --- |
| IoT Sensors | Real-time data collection | Precise moisture monitoring |
| Remote Sensing | Satellite/aerial imagery | Large-scale field assessment |
| AI Algorithms | Adaptive scheduling | Optimized water delivery times |
| Cloud Systems | Data storage/analysis | Historical trend tracking |
| Mobile Apps | Remote control/alerts | Real-time farmer advisories |
| Soil Sensors | Nutrient/moisture tracking | Reduces over-fertilization |
| Automated Valves | Precision flow control | Minimizes manual labor errors |
| GPS Mapping | Site-specific irrigation | Targeted zone management |

Principles of Operation

Tubing with emitters is placed underground, fed by low-pressure pumps; soil texture dictates depth to ensure capillary rise to roots. Advanced designs include flush lines for clog prevention and fertigation ports (Daraz et al., 2025)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlVtYXIiLCJmYW1pbHkiOiJEYXJheiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiLFoHRlZmFuIiwiZmFtaWx5IjoiQm9qbmVjIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllvdW5hcyIsImZhbWlseSI6IktoYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMDc3LTA0NzIiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzOTAvYWdyaWN1bHR1cmUxNTA1MDU1NCIsIm9wZW5hbGV4IjoiVzQ0MDgyMTk4MTEiLCJleGEiOiJodHRwczovL3d3dy5tZHBpLmNvbS8yMDc3LTA0NzIvMTUvNS81NTQifSwidGl0bGUiOiJFbmVyZ3ktRWZmaWNpZW50IFNtYXJ0IElycmlnYXRpb24gVGVjaG5vbG9naWVzOiBBIFBhdGh3YXkgdG8gV2F0ZXIgYW5kIEVuZXJneSBTdXN0YWluYWJpbGl0eSBpbiBBZ3JpY3VsdHVyZSIsImFic3RyYWN0IjoiVGhlIGFncmljdWx0dXJhbCBzZWN0b3IgZmFjZXMgY2hhbGxlbmdlcyBzdWNoIGFzIHdhdGVyIHNjYXJjaXR5LCBlbmVyZ3kgaW5lZmZpY2llbmN5LCBhbmQgZGVjbGluaW5nIHByb2R1Y3Rpdml0eSwgcGFydGljdWxhcmx5IGluIGFyaWQgcmVnaW9ucy4gVHJhZGl0aW9uYWwgaXJyaWdhdGlvbiBtZXRob2RzIGNvbnRyaWJ1dGUgdG8gcmVzb3VyY2UgZGVwbGV0aW9uIGFuZCBlbnZpcm9ubWVudGFsIGltcGFjdHMuIFNvbGFyLXBvd2VyZWQgc21hcnQgaXJyaWdhdGlvbiBzeXN0ZW1zIGludGVncmF0ZSBwcmVjaXNpb24gaXJyaWdhdGlvbiB3aXRoIHJlbmV3YWJsZSBlbmVyZ3ksIGltcHJvdmluZyB3YXRlciB1c2UgYW5kIHByb2R1Y3Rpdml0eS4gSW4gUGFraXN0YW4sIHdoZXJlIGFncmljdWx0dXJlIGNvbnRyaWJ1dGVzIDE5JSBvZiBncm9zcyBkb21lc3RpYyBwcm9kdWN0IGFuZCBlbXBsb3lzIDQwJSBvZiB0aGUgd29ya2ZvcmNlLCB0aGVzZSBjaGFsbGVuZ2VzIGFyZSBzZXZlcmUsIGVzcGVjaWFsbHkgaW4gd2F0ZXItc2NhcmNlIGFyZWFzIGxpa2UgdGhlIENob2xpc3RhbiBEZXNlcnQuIFRoaXMgc3R1ZHkgZXhhbWluZXMgdGhlIGltcGFjdCBvZiBzb2xhci1wb3dlcmVkIHNtYXJ0IGlycmlnYXRpb24gb24gYWdyaWN1bHR1cmFsIHByb2R1Y3Rpdml0eSwgd2F0ZXIgY29uc2VydmF0aW9uLCBhbmQgZW5lcmd5IGVmZmljaWVuY3kgaW4gdGhlIENob2xpc3RhbiBEZXNlcnQuIFVzaW5nIGEgcXVhbnRpdGF0aXZlIGNyb3NzLXNlY3Rpb25hbCBkZXNpZ24sIGRhdGEgd2VyZSBjb2xsZWN0ZWQgZnJvbSAzODQgZmFybWVycyB2aWEgc3RydWN0dXJlZCBxdWVzdGlvbm5haXJlcy4gU3RhdGlzdGljYWwgYW5hbHlzZXMsIGluY2x1ZGluZyBtdWx0aXBsZSBsaW5lYXIgcmVncmVzc2lvbiwgcGFpcmVkIHNhbXBsZSB0LXRlc3RzLCBhbmQgU3RydWN0dXJhbCBFcXVhdGlvbiBNb2RlbGluZyAoU0VNKSwgd2VyZSBjb25kdWN0ZWQuIEZpbmRpbmdzIHNob3cgc2lnbmlmaWNhbnQgaW1wcm92ZW1lbnRzIGluIGNyb3AgeWllbGQgKGZyb20gMy4wIHRvIDQuOCB0b25zL2hlY3RhcmUpIGFuZCByZWR1Y3Rpb25zIGluIHdhdGVyIGFuZCBlbmVyZ3kgY29uc3VtcHRpb24uIFJlZ3Jlc3Npb24gYW5hbHlzaXMgaGlnaGxpZ2h0ZWQgc3Ryb25nIHBvc2l0aXZlIGVmZmVjdHMgb24geWllbGQgYW5kIGVmZmljaWVuY3ksIHdoaWxlIFNFTSBjb25maXJtZWQgcmVkdWNlZCBlbnZpcm9ubWVudGFsIGltcGFjdCBhbmQgb3BlcmF0aW9uYWwgY29zdHMuIFRoZSBzdHVkeSBjb25jbHVkZXMgdGhhdCBzb2xhci1wb3dlcmVkIGlycmlnYXRpb24gZW5oYW5jZXMgcHJvZHVjdGl2aXR5LCBjb25zZXJ2ZXMgcmVzb3VyY2VzLCBhbmQgcHJvbW90ZXMgc3VzdGFpbmFiaWxpdHkuIFBvbGljeW1ha2VycyBzaG91bGQgcHJvdmlkZSBmaW5hbmNpYWwgaW5jZW50aXZlcywgaW52ZXN0IGluIHJlbmV3YWJsZSBpbmZyYXN0cnVjdHVyZSwgYW5kIGltcGxlbWVudCB0cmFpbmluZyBwcm9ncmFtcyB0byBzdXBwb3J0IGFkb3B0aW9uLiBDb2xsYWJvcmF0aXZlIGVmZm9ydHMgYXJlIGVzc2VudGlhbCBmb3Igc3VzdGFpbmFibGUgYWdyaWN1bHR1cmUgaW4gYXJpZCByZWdpb25zLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJpc3N1ZWREYXRlIjoiMjAyNS0wMy0wNSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkFncmljdWx0dXJlIiwiaXNzdWUiOjUsInZvbHVtZSI6MTUsImltcGFjdEZhY3RvciI6IjUuMjYifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjo1NTQsImxhc3RQYWdlIjo1NTR9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiTXVsdGlkaXNjaXBsaW5hcnkgRGlnaXRhbCBQdWJsaXNoaW5nIEluc3RpdHV0ZSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NDgsImNpdGVkQnlDb3VudCI6Mn19LCJxdW90ZSI6IjEuIEludHJvZHVjdGlvbiBUaGUgYWdyaWN1bHR1cmFsIHNlY3RvciwgYSBjb3JuZXJzdG9uZSBvZiBnbG9iYWwgZm9vZCBzZWN1cml0eSBhbmQgZWNvbm9taWMgZGV2ZWxvcG1lbnQsIGZhY2VzIG1vdW50aW5nIGNoYWxsZW5nZXMgaW4gdGhlIDIxc3QgY2VudHVyeS4gQ2xpbWF0ZSBjaGFuZ2UsIHJhcGlkIHBvcHVsYXRpb24gZ3Jvd3RoLCBhbmQgZHdpbmRsaW5nIG5hdHVyYWwgcmVzb3VyY2VzIGhhdmUgaW50ZW5zaWZpZWQgdGhlIG5lZWQgZm9yIHN1c3RhaW5hYmxlIGFncmljdWx0dXJhbCBwcmFjdGljZXMuIEFtb25nIHRoZXNlIGNoYWxsZW5nZXMsIHdhdGVyIHNjYXJjaXR5IGFuZCBlbmVyZ3kgaW5lZmZpY2llbmN5IHN0YW5kIG91dCBhcyBjcml0aWNhbCBiYXJyaWVycyB0byBwcm9kdWN0aXZpdHkgYW5kIHN1c3RhaW5hYmlsaXR5LiBHbG9iYWxseSwgYWdyaWN1bHR1cmUgYWNjb3VudHMgZm9yIGFwcHJveGltYXRlbHkgNzAlIG9mIGZyZXNod2F0ZXIgd2l0aGRyYXdhbHMgYW5kIGlzIG9uZSBvZiB0aGUgbGFyZ2VzdCBjb25zdW1lcnMgb2YgZW5lcmd5IGluIG1hbnkgcmVnaW9ucyBbIDEsIDJdLiBUcmFkaXRpb25hbCBpcnJpZ2F0aW9uIHByYWN0aWNlcywgb2Z0ZW4gaW5lZmZpY2llbnQgYW5kIHJlc291cmNlLWludGVuc2l2ZSwgZXhhY2VyYmF0ZSB0aGVzZSBpc3N1ZXMsIGxlYWRpbmcgdG8gd2F0ZXIgd2FzdGFnZSwgZW5lcmd5IG92ZXJ1c2UsIGFuZCByZWR1Y2VkIGNyb3AgeWllbGRzLiJ9XQ%3D%3D.

Advantages and Efficiency Gains

- Superior water conservation: Up to 90% savings, vital as agriculture drives 70% of freshwater use (Daraz et al., 2025)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlVtYXIiLCJmYW1pbHkiOiJEYXJheiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiLFoHRlZmFuIiwiZmFtaWx5IjoiQm9qbmVjIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllvdW5hcyIsImZhbWlseSI6IktoYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMDc3LTA0NzIiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzOTAvYWdyaWN1bHR1cmUxNTA1MDU1NCIsIm9wZW5hbGV4IjoiVzQ0MDgyMTk4MTEiLCJleGEiOiJodHRwczovL3d3dy5tZHBpLmNvbS8yMDc3LTA0NzIvMTUvNS81NTQifSwidGl0bGUiOiJFbmVyZ3ktRWZmaWNpZW50IFNtYXJ0IElycmlnYXRpb24gVGVjaG5vbG9naWVzOiBBIFBhdGh3YXkgdG8gV2F0ZXIgYW5kIEVuZXJneSBTdXN0YWluYWJpbGl0eSBpbiBBZ3JpY3VsdHVyZSIsImFic3RyYWN0IjoiVGhlIGFncmljdWx0dXJhbCBzZWN0b3IgZmFjZXMgY2hhbGxlbmdlcyBzdWNoIGFzIHdhdGVyIHNjYXJjaXR5LCBlbmVyZ3kgaW5lZmZpY2llbmN5LCBhbmQgZGVjbGluaW5nIHByb2R1Y3Rpdml0eSwgcGFydGljdWxhcmx5IGluIGFyaWQgcmVnaW9ucy4gVHJhZGl0aW9uYWwgaXJyaWdhdGlvbiBtZXRob2RzIGNvbnRyaWJ1dGUgdG8gcmVzb3VyY2UgZGVwbGV0aW9uIGFuZCBlbnZpcm9ubWVudGFsIGltcGFjdHMuIFNvbGFyLXBvd2VyZWQgc21hcnQgaXJyaWdhdGlvbiBzeXN0ZW1zIGludGVncmF0ZSBwcmVjaXNpb24gaXJyaWdhdGlvbiB3aXRoIHJlbmV3YWJsZSBlbmVyZ3ksIGltcHJvdmluZyB3YXRlciB1c2UgYW5kIHByb2R1Y3Rpdml0eS4gSW4gUGFraXN0YW4sIHdoZXJlIGFncmljdWx0dXJlIGNvbnRyaWJ1dGVzIDE5JSBvZiBncm9zcyBkb21lc3RpYyBwcm9kdWN0IGFuZCBlbXBsb3lzIDQwJSBvZiB0aGUgd29ya2ZvcmNlLCB0aGVzZSBjaGFsbGVuZ2VzIGFyZSBzZXZlcmUsIGVzcGVjaWFsbHkgaW4gd2F0ZXItc2NhcmNlIGFyZWFzIGxpa2UgdGhlIENob2xpc3RhbiBEZXNlcnQuIFRoaXMgc3R1ZHkgZXhhbWluZXMgdGhlIGltcGFjdCBvZiBzb2xhci1wb3dlcmVkIHNtYXJ0IGlycmlnYXRpb24gb24gYWdyaWN1bHR1cmFsIHByb2R1Y3Rpdml0eSwgd2F0ZXIgY29uc2VydmF0aW9uLCBhbmQgZW5lcmd5IGVmZmljaWVuY3kgaW4gdGhlIENob2xpc3RhbiBEZXNlcnQuIFVzaW5nIGEgcXVhbnRpdGF0aXZlIGNyb3NzLXNlY3Rpb25hbCBkZXNpZ24sIGRhdGEgd2VyZSBjb2xsZWN0ZWQgZnJvbSAzODQgZmFybWVycyB2aWEgc3RydWN0dXJlZCBxdWVzdGlvbm5haXJlcy4gU3RhdGlzdGljYWwgYW5hbHlzZXMsIGluY2x1ZGluZyBtdWx0aXBsZSBsaW5lYXIgcmVncmVzc2lvbiwgcGFpcmVkIHNhbXBsZSB0LXRlc3RzLCBhbmQgU3RydWN0dXJhbCBFcXVhdGlvbiBNb2RlbGluZyAoU0VNKSwgd2VyZSBjb25kdWN0ZWQuIEZpbmRpbmdzIHNob3cgc2lnbmlmaWNhbnQgaW1wcm92ZW1lbnRzIGluIGNyb3AgeWllbGQgKGZyb20gMy4wIHRvIDQuOCB0b25zL2hlY3RhcmUpIGFuZCByZWR1Y3Rpb25zIGluIHdhdGVyIGFuZCBlbmVyZ3kgY29uc3VtcHRpb24uIFJlZ3Jlc3Npb24gYW5hbHlzaXMgaGlnaGxpZ2h0ZWQgc3Ryb25nIHBvc2l0aXZlIGVmZmVjdHMgb24geWllbGQgYW5kIGVmZmljaWVuY3ksIHdoaWxlIFNFTSBjb25maXJtZWQgcmVkdWNlZCBlbnZpcm9ubWVudGFsIGltcGFjdCBhbmQgb3BlcmF0aW9uYWwgY29zdHMuIFRoZSBzdHVkeSBjb25jbHVkZXMgdGhhdCBzb2xhci1wb3dlcmVkIGlycmlnYXRpb24gZW5oYW5jZXMgcHJvZHVjdGl2aXR5LCBjb25zZXJ2ZXMgcmVzb3VyY2VzLCBhbmQgcHJvbW90ZXMgc3VzdGFpbmFiaWxpdHkuIFBvbGljeW1ha2VycyBzaG91bGQgcHJvdmlkZSBmaW5hbmNpYWwgaW5jZW50aXZlcywgaW52ZXN0IGluIHJlbmV3YWJsZSBpbmZyYXN0cnVjdHVyZSwgYW5kIGltcGxlbWVudCB0cmFpbmluZyBwcm9ncmFtcyB0byBzdXBwb3J0IGFkb3B0aW9uLiBDb2xsYWJvcmF0aXZlIGVmZm9ydHMgYXJlIGVzc2VudGlhbCBmb3Igc3VzdGFpbmFibGUgYWdyaWN1bHR1cmUgaW4gYXJpZCByZWdpb25zLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJpc3N1ZWREYXRlIjoiMjAyNS0wMy0wNSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkFncmljdWx0dXJlIiwiaXNzdWUiOjUsInZvbHVtZSI6MTUsImltcGFjdEZhY3RvciI6IjUuMjYifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjo1NTQsImxhc3RQYWdlIjo1NTR9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiTXVsdGlkaXNjaXBsaW5hcnkgRGlnaXRhbCBQdWJsaXNoaW5nIEluc3RpdHV0ZSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NDgsImNpdGVkQnlDb3VudCI6Mn19LCJxdW90ZSI6IjEuIEludHJvZHVjdGlvbiBUaGUgYWdyaWN1bHR1cmFsIHNlY3RvciwgYSBjb3JuZXJzdG9uZSBvZiBnbG9iYWwgZm9vZCBzZWN1cml0eSBhbmQgZWNvbm9taWMgZGV2ZWxvcG1lbnQsIGZhY2VzIG1vdW50aW5nIGNoYWxsZW5nZXMgaW4gdGhlIDIxc3QgY2VudHVyeS4gQ2xpbWF0ZSBjaGFuZ2UsIHJhcGlkIHBvcHVsYXRpb24gZ3Jvd3RoLCBhbmQgZHdpbmRsaW5nIG5hdHVyYWwgcmVzb3VyY2VzIGhhdmUgaW50ZW5zaWZpZWQgdGhlIG5lZWQgZm9yIHN1c3RhaW5hYmxlIGFncmljdWx0dXJhbCBwcmFjdGljZXMuIEFtb25nIHRoZXNlIGNoYWxsZW5nZXMsIHdhdGVyIHNjYXJjaXR5IGFuZCBlbmVyZ3kgaW5lZmZpY2llbmN5IHN0YW5kIG91dCBhcyBjcml0aWNhbCBiYXJyaWVycyB0byBwcm9kdWN0aXZpdHkgYW5kIHN1c3RhaW5hYmlsaXR5LiBHbG9iYWxseSwgYWdyaWN1bHR1cmUgYWNjb3VudHMgZm9yIGFwcHJveGltYXRlbHkgNzAlIG9mIGZyZXNod2F0ZXIgd2l0aGRyYXdhbHMgYW5kIGlzIG9uZSBvZiB0aGUgbGFyZ2VzdCBjb25zdW1lcnMgb2YgZW5lcmd5IGluIG1hbnkgcmVnaW9ucyBbIDEsIDJdLiBUcmFkaXRpb25hbCBpcnJpZ2F0aW9uIHByYWN0aWNlcywgb2Z0ZW4gaW5lZmZpY2llbnQgYW5kIHJlc291cmNlLWludGVuc2l2ZSwgZXhhY2VyYmF0ZSB0aGVzZSBpc3N1ZXMsIGxlYWRpbmcgdG8gd2F0ZXIgd2FzdGFnZSwgZW5lcmd5IG92ZXJ1c2UsIGFuZCByZWR1Y2VkIGNyb3AgeWllbGRzLiJ9XQ%3D%3D;

- Energy efficiency: Minimal pressure requirements reduce pumping costs (Daraz et al., 2025)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlVtYXIiLCJmYW1pbHkiOiJEYXJheiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiLFoHRlZmFuIiwiZmFtaWx5IjoiQm9qbmVjIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllvdW5hcyIsImZhbWlseSI6IktoYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMDc3LTA0NzIiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzOTAvYWdyaWN1bHR1cmUxNTA1MDU1NCIsIm9wZW5hbGV4IjoiVzQ0MDgyMTk4MTEiLCJleGEiOiJodHRwczovL3d3dy5tZHBpLmNvbS8yMDc3LTA0NzIvMTUvNS81NTQifSwidGl0bGUiOiJFbmVyZ3ktRWZmaWNpZW50IFNtYXJ0IElycmlnYXRpb24gVGVjaG5vbG9naWVzOiBBIFBhdGh3YXkgdG8gV2F0ZXIgYW5kIEVuZXJneSBTdXN0YWluYWJpbGl0eSBpbiBBZ3JpY3VsdHVyZSIsImFic3RyYWN0IjoiVGhlIGFncmljdWx0dXJhbCBzZWN0b3IgZmFjZXMgY2hhbGxlbmdlcyBzdWNoIGFzIHdhdGVyIHNjYXJjaXR5LCBlbmVyZ3kgaW5lZmZpY2llbmN5LCBhbmQgZGVjbGluaW5nIHByb2R1Y3Rpdml0eSwgcGFydGljdWxhcmx5IGluIGFyaWQgcmVnaW9ucy4gVHJhZGl0aW9uYWwgaXJyaWdhdGlvbiBtZXRob2RzIGNvbnRyaWJ1dGUgdG8gcmVzb3VyY2UgZGVwbGV0aW9uIGFuZCBlbnZpcm9ubWVudGFsIGltcGFjdHMuIFNvbGFyLXBvd2VyZWQgc21hcnQgaXJyaWdhdGlvbiBzeXN0ZW1zIGludGVncmF0ZSBwcmVjaXNpb24gaXJyaWdhdGlvbiB3aXRoIHJlbmV3YWJsZSBlbmVyZ3ksIGltcHJvdmluZyB3YXRlciB1c2UgYW5kIHByb2R1Y3Rpdml0eS4gSW4gUGFraXN0YW4sIHdoZXJlIGFncmljdWx0dXJlIGNvbnRyaWJ1dGVzIDE5JSBvZiBncm9zcyBkb21lc3RpYyBwcm9kdWN0IGFuZCBlbXBsb3lzIDQwJSBvZiB0aGUgd29ya2ZvcmNlLCB0aGVzZSBjaGFsbGVuZ2VzIGFyZSBzZXZlcmUsIGVzcGVjaWFsbHkgaW4gd2F0ZXItc2NhcmNlIGFyZWFzIGxpa2UgdGhlIENob2xpc3RhbiBEZXNlcnQuIFRoaXMgc3R1ZHkgZXhhbWluZXMgdGhlIGltcGFjdCBvZiBzb2xhci1wb3dlcmVkIHNtYXJ0IGlycmlnYXRpb24gb24gYWdyaWN1bHR1cmFsIHByb2R1Y3Rpdml0eSwgd2F0ZXIgY29uc2VydmF0aW9uLCBhbmQgZW5lcmd5IGVmZmljaWVuY3kgaW4gdGhlIENob2xpc3RhbiBEZXNlcnQuIFVzaW5nIGEgcXVhbnRpdGF0aXZlIGNyb3NzLXNlY3Rpb25hbCBkZXNpZ24sIGRhdGEgd2VyZSBjb2xsZWN0ZWQgZnJvbSAzODQgZmFybWVycyB2aWEgc3RydWN0dXJlZCBxdWVzdGlvbm5haXJlcy4gU3RhdGlzdGljYWwgYW5hbHlzZXMsIGluY2x1ZGluZyBtdWx0aXBsZSBsaW5lYXIgcmVncmVzc2lvbiwgcGFpcmVkIHNhbXBsZSB0LXRlc3RzLCBhbmQgU3RydWN0dXJhbCBFcXVhdGlvbiBNb2RlbGluZyAoU0VNKSwgd2VyZSBjb25kdWN0ZWQuIEZpbmRpbmdzIHNob3cgc2lnbmlmaWNhbnQgaW1wcm92ZW1lbnRzIGluIGNyb3AgeWllbGQgKGZyb20gMy4wIHRvIDQuOCB0b25zL2hlY3RhcmUpIGFuZCByZWR1Y3Rpb25zIGluIHdhdGVyIGFuZCBlbmVyZ3kgY29uc3VtcHRpb24uIFJlZ3Jlc3Npb24gYW5hbHlzaXMgaGlnaGxpZ2h0ZWQgc3Ryb25nIHBvc2l0aXZlIGVmZmVjdHMgb24geWllbGQgYW5kIGVmZmljaWVuY3ksIHdoaWxlIFNFTSBjb25maXJtZWQgcmVkdWNlZCBlbnZpcm9ubWVudGFsIGltcGFjdCBhbmQgb3BlcmF0aW9uYWwgY29zdHMuIFRoZSBzdHVkeSBjb25jbHVkZXMgdGhhdCBzb2xhci1wb3dlcmVkIGlycmlnYXRpb24gZW5oYW5jZXMgcHJvZHVjdGl2aXR5LCBjb25zZXJ2ZXMgcmVzb3VyY2VzLCBhbmQgcHJvbW90ZXMgc3VzdGFpbmFiaWxpdHkuIFBvbGljeW1ha2VycyBzaG91bGQgcHJvdmlkZSBmaW5hbmNpYWwgaW5jZW50aXZlcywgaW52ZXN0IGluIHJlbmV3YWJsZSBpbmZyYXN0cnVjdHVyZSwgYW5kIGltcGxlbWVudCB0cmFpbmluZyBwcm9ncmFtcyB0byBzdXBwb3J0IGFkb3B0aW9uLiBDb2xsYWJvcmF0aXZlIGVmZm9ydHMgYXJlIGVzc2VudGlhbCBmb3Igc3VzdGFpbmFibGUgYWdyaWN1bHR1cmUgaW4gYXJpZCByZWdpb25zLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJpc3N1ZWREYXRlIjoiMjAyNS0wMy0wNSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkFncmljdWx0dXJlIiwiaXNzdWUiOjUsInZvbHVtZSI6MTUsImltcGFjdEZhY3RvciI6IjUuMjYifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjo1NTQsImxhc3RQYWdlIjo1NTR9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiTXVsdGlkaXNjaXBsaW5hcnkgRGlnaXRhbCBQdWJsaXNoaW5nIEluc3RpdHV0ZSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NDgsImNpdGVkQnlDb3VudCI6Mn19LCJxdW90ZSI6IjEuIEludHJvZHVjdGlvbiBUaGUgYWdyaWN1bHR1cmFsIHNlY3RvciwgYSBjb3JuZXJzdG9uZSBvZiBnbG9iYWwgZm9vZCBzZWN1cml0eSBhbmQgZWNvbm9taWMgZGV2ZWxvcG1lbnQsIGZhY2VzIG1vdW50aW5nIGNoYWxsZW5nZXMgaW4gdGhlIDIxc3QgY2VudHVyeS4gQ2xpbWF0ZSBjaGFuZ2UsIHJhcGlkIHBvcHVsYXRpb24gZ3Jvd3RoLCBhbmQgZHdpbmRsaW5nIG5hdHVyYWwgcmVzb3VyY2VzIGhhdmUgaW50ZW5zaWZpZWQgdGhlIG5lZWQgZm9yIHN1c3RhaW5hYmxlIGFncmljdWx0dXJhbCBwcmFjdGljZXMuIEFtb25nIHRoZXNlIGNoYWxsZW5nZXMsIHdhdGVyIHNjYXJjaXR5IGFuZCBlbmVyZ3kgaW5lZmZpY2llbmN5IHN0YW5kIG91dCBhcyBjcml0aWNhbCBiYXJyaWVycyB0byBwcm9kdWN0aXZpdHkgYW5kIHN1c3RhaW5hYmlsaXR5LiBHbG9iYWxseSwgYWdyaWN1bHR1cmUgYWNjb3VudHMgZm9yIGFwcHJveGltYXRlbHkgNzAlIG9mIGZyZXNod2F0ZXIgd2l0aGRyYXdhbHMgYW5kIGlzIG9uZSBvZiB0aGUgbGFyZ2VzdCBjb25zdW1lcnMgb2YgZW5lcmd5IGluIG1hbnkgcmVnaW9ucyBbIDEsIDJdLiBUcmFkaXRpb25hbCBpcnJpZ2F0aW9uIHByYWN0aWNlcywgb2Z0ZW4gaW5lZmZpY2llbnQgYW5kIHJlc291cmNlLWludGVuc2l2ZSwgZXhhY2VyYmF0ZSB0aGVzZSBpc3N1ZXMsIGxlYWRpbmcgdG8gd2F0ZXIgd2FzdGFnZSwgZW5lcmd5IG92ZXJ1c2UsIGFuZCByZWR1Y2VkIGNyb3AgeWllbGRzLiJ9XQ%3D%3D;

- Crop benefits: Deeper rooting, reduced evaporation, and no surface interference boost yields (Jumintono et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikp1bWludG9ubyIsImZhbWlseSI6Ikp1bWludG9ubyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJOdXJuYXdhdHkiLCJmYW1pbHkiOiJOdXJuYXdhdHkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTnVncm9obyBUcmlzbnUiLCJmYW1pbHkiOiJCcmF0YSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJEaW5pIiwiZmFtaWx5IjoiUmFobWlhdGkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSGFkaSIsImZhbWlseSI6IlN1c2lsbyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBLiBBLiIsImZhbWlseSI6Ikh1c3NlbiJ9fV0sImlkcyI6eyJpc3NuIjpbIjE0MjktNzQyNiIsIjIwODMtNDUzNSJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMjQ0MjUvandsZC4yMDI0LjE1MTc5OCIsIm9wZW5hbGV4IjoiVzQ0MDU0NzcxMDAifSwidGl0bGUiOiJDb21wYXJhdGl2ZSBhbmFseXNpcyBvZiBwcmVjaXNpb24sIGRlZmljaXQsIGFuZCBkcmlwIGlycmlnYXRpb24gZm9yIHdhdGVyIGVmZmljaWVuY3kgaW4gSXJhcSIsImFic3RyYWN0IjoiU3VzdGFpbmFibGUgaXJyaWdhdGlvbiBwcmFjdGljZXMgYXJlIGNyaXRpY2FsIGZvciBhZGRyZXNzaW5nIHRoZSBjaGFsbGVuZ2VzIG9mIHdhdGVyIHNjYXJjaXR5IGFuZCBmb29kIGluc2VjdXJpdHkgaW4gdGhlIGZhY2Ugb2YgY2xpbWF0ZSBjaGFuZ2UgYW5kIGdyb3dpbmcgZ2xvYmFsIGZvb2QgZGVtYW5kLiBUaGlzIHN0dWR5IGludmVzdGlnYXRlZCB0aGUgcG90ZW50aWFsIG9mIHByZWNpc2lvbiBpcnJpZ2F0aW9uIChQSSksIGRlZmljaXQgaXJyaWdhdGlvbiAoREkpLCBhbmQgZHJpcCBpcnJpZ2F0aW9uIChEUkkpIHRlY2huaXF1ZXMgdG8gZW5oYW5jZSBjcm9wIHByb2R1Y3Rpdml0eSBhbmQgd2F0ZXIgdXNlIGVmZmljaWVuY3kgKFdVRSkgY29tcGFyZWQgdG8gY29udmVudGlvbmFsIGZsb29kIGlycmlnYXRpb24gKENGSSkgaW4gdGhyZWUgZGlzdGluY3QgYWdyb2Vjb2xvZ2ljYWwgem9uZXMgb2YgSXJhcS4gRmllbGQgZXhwZXJpbWVudHMgd2VyZSBjb25kdWN0ZWQgdXNpbmcgYSByYW5kb21pc2VkIGNvbXBsZXRlIGJsb2NrIGRlc2lnbiB3aXRoIHdoZWF0LCBtYWl6ZSwgYW5kIHJpY2UgY3JvcHMuIFRoZSByZXN1bHRzIGRlbW9uc3RyYXRlZCB0aGF0IFBJIHNpZ25pZmljYW50bHkgaW5jcmVhc2VkIGNyb3AgZ3Jvd3RoIHBhcmFtZXRlcnMsIGdyYWluIHlpZWxkLCBhbmQgV1VFIGFjcm9zcyBhbGwgem9uZXMsIHdpdGggeWllbGQgaW1wcm92ZW1lbnRzIG9mIDMz4oCTMzglIGFuZCBXVUUgaW5jcmVhc2VzIG9mIDQ24oCTNTElIGluIGNvbnRyYXN0IHdpdGggQ0ZJLiBUaGUgREkgYW5kIERSSSB0cmVhdG1lbnRzIGFsc28gb3V0cGVyZm9ybWVkIENGSSwgYWxiZWl0IHRvIGEgbGVzc2VyIGV4dGVudC4gUmVtb3RlIHNlbnNpbmctZGVyaXZlZCB2ZWdldGF0aW9uIGluZGljZXMgc3Ryb25nbHkgY29ycmVsYXRlZCB3aXRoIGNyb3AgZ3Jvd3RoIHBhcmFtZXRlcnMgYW5kIHlpZWxkLCB3aGlsZSBoeWRyb2xvZ2ljYWwgbW9kZWxsaW5nIHJldmVhbGVkIHJlZHVjZWQgZXZhcG90cmFuc3BpcmF0aW9uIGFuZCBzdXJmYWNlIHJ1bm9mZiB1bmRlciB0aGUgUEkgdHJlYXRtZW50LiBUaGUgc3VzdGFpbmFibGUgaXJyaWdhdGlvbiBwcmFjdGljZXMgcmVzdWx0ZWQgaW4gc3Vic3RhbnRpYWwgd2F0ZXIgc2F2aW5ncyBvZiAyMOKAkzMwJSBjb21wYXJlZCB0byBDRkkuIFRoZXNlIGZpbmRpbmdzIGhpZ2hsaWdodCB0aGUgaW1wb3J0YW5jZSBvZiBhZG9wdGluZyBlZmZpY2llbnQgaXJyaWdhdGlvbiB0ZWNobmlxdWVzLCBhbG9uZyB3aXRoIGEgaG9saXN0aWMgYXBwcm9hY2ggZW5jb21wYXNzaW5nIHRlY2hub2xvZ2ljYWwgaW5ub3ZhdGlvbnMsIGNhcGFjaXR5IGJ1aWxkaW5nLCBhbmQgc3Rha2Vob2xkZXIgZW5nYWdlbWVudCwgdG8gcHJvbW90ZSB3YXRlci0gZWZmaWNpZW50IGFncmljdWx0dXJlIGFuZCBlbnN1cmUgZm9vZCBzZWN1cml0eSBpbiB3YXRlci1zY2FyY2UgcmVnaW9ucy4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjQ0MjUvandsZC4yMDI0LjE1MTc5OCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjI0NDI1L2p3bGQuMjAyNC4xNTE3OTgiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMi0xNCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkpvdXJuYWwgb2YgV2F0ZXIgYW5kIExhbmQgRGV2ZWxvcG1lbnQiLCJpbXBhY3RGYWN0b3IiOiIxLjQ2In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTMwLCJsYXN0UGFnZSI6MTM2fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkRlIEdydXl0ZXIgT3BlbiJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MzIsImNpdGVkQnlDb3VudCI6MX19LCJxdW90ZSI6IkluIHRoaXMgY29udGV4dCwgdGhlIGFkb3B0aW9uIG9mICBzdXN0YWluYWJsZSBpcnJpZ2F0aW9uIHByYWN0aWNlcyBoYXMgZW1lcmdlZCBhcyBhIGNyaXRpY2FsIHN0cmF0ZWd5ICBmb3IgZW5oYW5jaW5nIGZvb2Qgc2VjdXJpdHkgd2hpbGUgcHJvbW90aW5nIHdhdGVyIGNvbnNlcnZhdGlvbiAgaW4gYWdyaWN1bHR1cmFsIGxhbmRzY2FwZXMuICBJcnJpZ2F0aW9uIHBsYXlzIGEgdml0YWwgcm9sZSBpbiBnbG9iYWwgZm9vZCBwcm9kdWN0aW9uLCB3aXRoICBpcnJpZ2F0ZWQgYWdyaWN1bHR1cmUgYWNjb3VudGluZyBmb3IgYXBwcm94aW1hdGVseSA0MCUgb2YgdGhlICBmb29kIHN1cHBseSAoV3JhY2hpZW4gZGUsIFNjaHVsdHogYW5kIEdvbGksIDIwMjE7IFNjaG1pdHQsIFJvc2EgIGFuZCBEYWlseSwgMjAyMikuIEhvd2V2ZXIsIHRyYWRpdGlvbmFsIGlycmlnYXRpb24gbWV0aG9kcywgc3VjaCBhcyAgZmxvb2QgYW5kIGZ1cnJvdyBpcnJpZ2F0aW9uLCBhcmUgb2Z0ZW4gaW5lZmZpY2llbnQsIGxlYWRpbmcgdG8gIHN1YnN0YW50aWFsIHdhdGVyIGxvc3NlcyB0aHJvdWdoIGV2YXBvcmF0aW9uLCBydW5vZmYsIGFuZCBkZWVwICBwZXJjb2xhdGlvbiAoQW1ib21zYSwgMjAyMCkuIE1vcmVvdmVyLCB0aGUgZXhjZXNzaXZlIGFwcGxpY2EtdGlvbiBvZiB3YXRlciBjYW4gcmVzdWx0IGluIHNvaWwgc2FsaW5pc2F0aW9uLCBudXRyaWVudCBsZWFjaGluZywgYW5kICBncm91bmR3YXRlciBkZXBsZXRpb24sIGZ1cnRoZXIgY29tcHJvbWlzaW5nIHRoZSBsb25nLXRlcm0gIHN1c3RhaW5hYmlsaXR5IG9mIGFncmljdWx0dXJhbCBzeXN0ZW1zIChZaW4gZXQgYWwuLCAyMDIyKS4gVG8gYWRkcmVzcyAgdGhlc2UgY2hhbGxlbmdlcywgcmVzZWFyY2hlcnMgYW5kIHByYWN0aXRpb25lcnMgaGF2ZSBiZWVuIGV4cGxvci1pbmcgaW5ub3ZhdGl2ZSBpcnJpZ2F0aW9uIHRlY2huaXF1ZXMgYW5kIHdhdGVyIG1hbmFnZW1lbnQgIHN0cmF0ZWdpZXMgdGhhdCBjYW4gb3B0aW1pc2Ugd2F0ZXIgdXNlIGVmZmljaWVuY3kgKFdVRSkgd2hpbGUgIG1haW50YWluaW5nIG9yIGVuaGFuY2luZyBjcm9wIHByb2R1Y3Rpdml0eS4gIFByZWNpc2lvbiBpcnJpZ2F0aW9uIChQSSkgb2ZmZXJzIGEgcHJvbWlzaW5nIHNvbHV0aW9uLCAgaW52b2x2aW5nIHRoZSB0YXJnZXRlZCBhcHBsaWNhdGlvbiBvZiB3YXRlciBzcGVjaWZpY2FsbHkgdG8gdGhlICBuZWVkcyBvZiBpbmRpdmlkdWFsIHBsYW50cyBvciBjcm9wIHpvbmVzIChaaW5rZXJuYWdlbCBldCBhbC4sIDIwMjApLiAgIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyMi44ODY3MzU5MTYxMzc2OTUsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzQ2MjIxOTIzODI4MTI1LCJoZWlnaHQiOjEuNDUwNTQxMDE5NDM5Njk3M319LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI0LjM1MTEyMzgwOTgxNDQ1MywibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTI3MTYwNjQ0NSwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI1LjcyNDk4NzAzMDAyOTI5NywibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTY1MzA3NjE3MiwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI3LjA5ODg1MDI1MDI0NDE0LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjE2LjQxODE2MTM5MjIxMTkxNCwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI4LjQ3MjcwOTY1NTc2MTcyLCJsZWZ0IjoxMi4zODA4ODAzNTU4MzQ5NjEsIndpZHRoIjozNy4wMTIxODQxNDMwNjY0MDYsImhlaWdodCI6MS4zNjAwMTQ3OTYyNTcwMTl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyOS44NDY1NzI4NzU5NzY1NjIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQwNjQ4NjUxMTIzMDUsImhlaWdodCI6MS4zNjAwMTY3MDM2MDU2NTE5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzEuMjE0MDM2OTQxNTI4MzIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQ0ODU2MjYyMjA3MDMsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzIuNTg3OTAyMDY5MDkxOCwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDYwNTc4OTE4NDU3LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMzLjk2MTc2MTQ3NDYwOTM3NSwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDA0ODQ2MTkxNDA2MjUsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzUuMzM1NjI0Njk0ODI0MjIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzM5NzQ0NTY3ODcxMDk0LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjM2LjcwOTQ4NzkxNTAzOTA2LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjAwMTU5MDcyODc1OTc2NiwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC4wODMzNDczMjA1NTY2NCwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTY1MzA3NjE3MiwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOS40NTcyMTA1NDA3NzE0ODQsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQzMDQ4MDk1NzAzMTI1LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQwLjc0MDU0NzE4MDE3NTc4LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjM0NTQ1NTE2OTY3NzczNCwiaGVpZ2h0IjoxLjQ1MDU0MjgwNzU3OTA0MDV9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Mi4yMDQ5MzMxNjY1MDM5MDYsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMDAzMjUwMTIyMDcwMzEsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDMuNTc4Nzk2Mzg2NzE4NzUsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzM3OTIxMTQyNTc4MTI1LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQ0Ljg2MjEzMzAyNjEyMzA1LCJsZWZ0Ijo5LjA0NzYwMTY5OTgyOTEwMiwid2lkdGgiOjQwLjM0NTMxNDAyNTg3ODkwNiwiaGVpZ2h0IjoxLjQ1MDU0MjgwNzU3OTA0MDV9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ni4zMjY1MTkwMTI0NTExNywibGVmdCI6OS4wNDc2MDE2OTk4MjkxMDIsIndpZHRoIjoyNy45Njc0NDcyODA4ODM3OSwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ny43MDAzODIyMzI2NjYwMTYsImxlZnQiOjEyLjM4MDg3ODQ0ODQ4NjMyOCwid2lkdGgiOjM3LjAzNDc4NjIyNDM2NTIzNCwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0OS4wNzQyNDU0NTI4ODA4NiwibGVmdCI6OS4wNDc2MDE2OTk4MjkxMDIsIndpZHRoIjo0MC4zNDA2NDQ4MzY0MjU3OCwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1MC4zNTc1ODIwOTIyODUxNTYsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzQ0ODUyNDQ3NTA5NzY2LCJoZWlnaHQiOjEuNDUwNTQyODA3NTc5MDQwNX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEzMSJ9fV0%3D;

- Environmental gains: Limits weed growth, evaporation, and nutrient runoff.

## Smart Irrigation Systems and Precision Agriculture

Smart irrigation systems leverage sensors, data analytics, and automation for real-time, site-specific water management, revolutionizing precision agriculture by integrating with drip, sprinkler, and pivot technologies to achieve dynamic WUE gains of 20-40% beyond static methods (Jumintono et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikp1bWludG9ubyIsImZhbWlseSI6Ikp1bWludG9ubyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJOdXJuYXdhdHkiLCJmYW1pbHkiOiJOdXJuYXdhdHkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTnVncm9obyBUcmlzbnUiLCJmYW1pbHkiOiJCcmF0YSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJEaW5pIiwiZmFtaWx5IjoiUmFobWlhdGkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSGFkaSIsImZhbWlseSI6IlN1c2lsbyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBLiBBLiIsImZhbWlseSI6Ikh1c3NlbiJ9fV0sImlkcyI6eyJpc3NuIjpbIjE0MjktNzQyNiIsIjIwODMtNDUzNSJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMjQ0MjUvandsZC4yMDI0LjE1MTc5OCIsIm9wZW5hbGV4IjoiVzQ0MDU0NzcxMDAifSwidGl0bGUiOiJDb21wYXJhdGl2ZSBhbmFseXNpcyBvZiBwcmVjaXNpb24sIGRlZmljaXQsIGFuZCBkcmlwIGlycmlnYXRpb24gZm9yIHdhdGVyIGVmZmljaWVuY3kgaW4gSXJhcSIsImFic3RyYWN0IjoiU3VzdGFpbmFibGUgaXJyaWdhdGlvbiBwcmFjdGljZXMgYXJlIGNyaXRpY2FsIGZvciBhZGRyZXNzaW5nIHRoZSBjaGFsbGVuZ2VzIG9mIHdhdGVyIHNjYXJjaXR5IGFuZCBmb29kIGluc2VjdXJpdHkgaW4gdGhlIGZhY2Ugb2YgY2xpbWF0ZSBjaGFuZ2UgYW5kIGdyb3dpbmcgZ2xvYmFsIGZvb2QgZGVtYW5kLiBUaGlzIHN0dWR5IGludmVzdGlnYXRlZCB0aGUgcG90ZW50aWFsIG9mIHByZWNpc2lvbiBpcnJpZ2F0aW9uIChQSSksIGRlZmljaXQgaXJyaWdhdGlvbiAoREkpLCBhbmQgZHJpcCBpcnJpZ2F0aW9uIChEUkkpIHRlY2huaXF1ZXMgdG8gZW5oYW5jZSBjcm9wIHByb2R1Y3Rpdml0eSBhbmQgd2F0ZXIgdXNlIGVmZmljaWVuY3kgKFdVRSkgY29tcGFyZWQgdG8gY29udmVudGlvbmFsIGZsb29kIGlycmlnYXRpb24gKENGSSkgaW4gdGhyZWUgZGlzdGluY3QgYWdyb2Vjb2xvZ2ljYWwgem9uZXMgb2YgSXJhcS4gRmllbGQgZXhwZXJpbWVudHMgd2VyZSBjb25kdWN0ZWQgdXNpbmcgYSByYW5kb21pc2VkIGNvbXBsZXRlIGJsb2NrIGRlc2lnbiB3aXRoIHdoZWF0LCBtYWl6ZSwgYW5kIHJpY2UgY3JvcHMuIFRoZSByZXN1bHRzIGRlbW9uc3RyYXRlZCB0aGF0IFBJIHNpZ25pZmljYW50bHkgaW5jcmVhc2VkIGNyb3AgZ3Jvd3RoIHBhcmFtZXRlcnMsIGdyYWluIHlpZWxkLCBhbmQgV1VFIGFjcm9zcyBhbGwgem9uZXMsIHdpdGggeWllbGQgaW1wcm92ZW1lbnRzIG9mIDMz4oCTMzglIGFuZCBXVUUgaW5jcmVhc2VzIG9mIDQ24oCTNTElIGluIGNvbnRyYXN0IHdpdGggQ0ZJLiBUaGUgREkgYW5kIERSSSB0cmVhdG1lbnRzIGFsc28gb3V0cGVyZm9ybWVkIENGSSwgYWxiZWl0IHRvIGEgbGVzc2VyIGV4dGVudC4gUmVtb3RlIHNlbnNpbmctZGVyaXZlZCB2ZWdldGF0aW9uIGluZGljZXMgc3Ryb25nbHkgY29ycmVsYXRlZCB3aXRoIGNyb3AgZ3Jvd3RoIHBhcmFtZXRlcnMgYW5kIHlpZWxkLCB3aGlsZSBoeWRyb2xvZ2ljYWwgbW9kZWxsaW5nIHJldmVhbGVkIHJlZHVjZWQgZXZhcG90cmFuc3BpcmF0aW9uIGFuZCBzdXJmYWNlIHJ1bm9mZiB1bmRlciB0aGUgUEkgdHJlYXRtZW50LiBUaGUgc3VzdGFpbmFibGUgaXJyaWdhdGlvbiBwcmFjdGljZXMgcmVzdWx0ZWQgaW4gc3Vic3RhbnRpYWwgd2F0ZXIgc2F2aW5ncyBvZiAyMOKAkzMwJSBjb21wYXJlZCB0byBDRkkuIFRoZXNlIGZpbmRpbmdzIGhpZ2hsaWdodCB0aGUgaW1wb3J0YW5jZSBvZiBhZG9wdGluZyBlZmZpY2llbnQgaXJyaWdhdGlvbiB0ZWNobmlxdWVzLCBhbG9uZyB3aXRoIGEgaG9saXN0aWMgYXBwcm9hY2ggZW5jb21wYXNzaW5nIHRlY2hub2xvZ2ljYWwgaW5ub3ZhdGlvbnMsIGNhcGFjaXR5IGJ1aWxkaW5nLCBhbmQgc3Rha2Vob2xkZXIgZW5nYWdlbWVudCwgdG8gcHJvbW90ZSB3YXRlci0gZWZmaWNpZW50IGFncmljdWx0dXJlIGFuZCBlbnN1cmUgZm9vZCBzZWN1cml0eSBpbiB3YXRlci1zY2FyY2UgcmVnaW9ucy4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjQ0MjUvandsZC4yMDI0LjE1MTc5OCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjI0NDI1L2p3bGQuMjAyNC4xNTE3OTgiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMi0xNCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkpvdXJuYWwgb2YgV2F0ZXIgYW5kIExhbmQgRGV2ZWxvcG1lbnQiLCJpbXBhY3RGYWN0b3IiOiIxLjQ2In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTMwLCJsYXN0UGFnZSI6MTM2fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkRlIEdydXl0ZXIgT3BlbiJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MzIsImNpdGVkQnlDb3VudCI6MX19LCJxdW90ZSI6IkluIHRoaXMgY29udGV4dCwgdGhlIGFkb3B0aW9uIG9mICBzdXN0YWluYWJsZSBpcnJpZ2F0aW9uIHByYWN0aWNlcyBoYXMgZW1lcmdlZCBhcyBhIGNyaXRpY2FsIHN0cmF0ZWd5ICBmb3IgZW5oYW5jaW5nIGZvb2Qgc2VjdXJpdHkgd2hpbGUgcHJvbW90aW5nIHdhdGVyIGNvbnNlcnZhdGlvbiAgaW4gYWdyaWN1bHR1cmFsIGxhbmRzY2FwZXMuICBJcnJpZ2F0aW9uIHBsYXlzIGEgdml0YWwgcm9sZSBpbiBnbG9iYWwgZm9vZCBwcm9kdWN0aW9uLCB3aXRoICBpcnJpZ2F0ZWQgYWdyaWN1bHR1cmUgYWNjb3VudGluZyBmb3IgYXBwcm94aW1hdGVseSA0MCUgb2YgdGhlICBmb29kIHN1cHBseSAoV3JhY2hpZW4gZGUsIFNjaHVsdHogYW5kIEdvbGksIDIwMjE7IFNjaG1pdHQsIFJvc2EgIGFuZCBEYWlseSwgMjAyMikuIEhvd2V2ZXIsIHRyYWRpdGlvbmFsIGlycmlnYXRpb24gbWV0aG9kcywgc3VjaCBhcyAgZmxvb2QgYW5kIGZ1cnJvdyBpcnJpZ2F0aW9uLCBhcmUgb2Z0ZW4gaW5lZmZpY2llbnQsIGxlYWRpbmcgdG8gIHN1YnN0YW50aWFsIHdhdGVyIGxvc3NlcyB0aHJvdWdoIGV2YXBvcmF0aW9uLCBydW5vZmYsIGFuZCBkZWVwICBwZXJjb2xhdGlvbiAoQW1ib21zYSwgMjAyMCkuIE1vcmVvdmVyLCB0aGUgZXhjZXNzaXZlIGFwcGxpY2EtdGlvbiBvZiB3YXRlciBjYW4gcmVzdWx0IGluIHNvaWwgc2FsaW5pc2F0aW9uLCBudXRyaWVudCBsZWFjaGluZywgYW5kICBncm91bmR3YXRlciBkZXBsZXRpb24sIGZ1cnRoZXIgY29tcHJvbWlzaW5nIHRoZSBsb25nLXRlcm0gIHN1c3RhaW5hYmlsaXR5IG9mIGFncmljdWx0dXJhbCBzeXN0ZW1zIChZaW4gZXQgYWwuLCAyMDIyKS4gVG8gYWRkcmVzcyAgdGhlc2UgY2hhbGxlbmdlcywgcmVzZWFyY2hlcnMgYW5kIHByYWN0aXRpb25lcnMgaGF2ZSBiZWVuIGV4cGxvci1pbmcgaW5ub3ZhdGl2ZSBpcnJpZ2F0aW9uIHRlY2huaXF1ZXMgYW5kIHdhdGVyIG1hbmFnZW1lbnQgIHN0cmF0ZWdpZXMgdGhhdCBjYW4gb3B0aW1pc2Ugd2F0ZXIgdXNlIGVmZmljaWVuY3kgKFdVRSkgd2hpbGUgIG1haW50YWluaW5nIG9yIGVuaGFuY2luZyBjcm9wIHByb2R1Y3Rpdml0eS4gIFByZWNpc2lvbiBpcnJpZ2F0aW9uIChQSSkgb2ZmZXJzIGEgcHJvbWlzaW5nIHNvbHV0aW9uLCAgaW52b2x2aW5nIHRoZSB0YXJnZXRlZCBhcHBsaWNhdGlvbiBvZiB3YXRlciBzcGVjaWZpY2FsbHkgdG8gdGhlICBuZWVkcyBvZiBpbmRpdmlkdWFsIHBsYW50cyBvciBjcm9wIHpvbmVzIChaaW5rZXJuYWdlbCBldCBhbC4sIDIwMjApLiAgIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyMi44ODY3MzU5MTYxMzc2OTUsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzQ2MjIxOTIzODI4MTI1LCJoZWlnaHQiOjEuNDUwNTQxMDE5NDM5Njk3M319LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI0LjM1MTEyMzgwOTgxNDQ1MywibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTI3MTYwNjQ0NSwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI1LjcyNDk4NzAzMDAyOTI5NywibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTY1MzA3NjE3MiwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI3LjA5ODg1MDI1MDI0NDE0LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjE2LjQxODE2MTM5MjIxMTkxNCwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI4LjQ3MjcwOTY1NTc2MTcyLCJsZWZ0IjoxMi4zODA4ODAzNTU4MzQ5NjEsIndpZHRoIjozNy4wMTIxODQxNDMwNjY0MDYsImhlaWdodCI6MS4zNjAwMTQ3OTYyNTcwMTl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyOS44NDY1NzI4NzU5NzY1NjIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQwNjQ4NjUxMTIzMDUsImhlaWdodCI6MS4zNjAwMTY3MDM2MDU2NTE5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzEuMjE0MDM2OTQxNTI4MzIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQ0ODU2MjYyMjA3MDMsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzIuNTg3OTAyMDY5MDkxOCwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDYwNTc4OTE4NDU3LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMzLjk2MTc2MTQ3NDYwOTM3NSwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDA0ODQ2MTkxNDA2MjUsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzUuMzM1NjI0Njk0ODI0MjIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzM5NzQ0NTY3ODcxMDk0LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjM2LjcwOTQ4NzkxNTAzOTA2LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjAwMTU5MDcyODc1OTc2NiwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC4wODMzNDczMjA1NTY2NCwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTY1MzA3NjE3MiwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOS40NTcyMTA1NDA3NzE0ODQsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQzMDQ4MDk1NzAzMTI1LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQwLjc0MDU0NzE4MDE3NTc4LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjM0NTQ1NTE2OTY3NzczNCwiaGVpZ2h0IjoxLjQ1MDU0MjgwNzU3OTA0MDV9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Mi4yMDQ5MzMxNjY1MDM5MDYsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMDAzMjUwMTIyMDcwMzEsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDMuNTc4Nzk2Mzg2NzE4NzUsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzM3OTIxMTQyNTc4MTI1LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQ0Ljg2MjEzMzAyNjEyMzA1LCJsZWZ0Ijo5LjA0NzYwMTY5OTgyOTEwMiwid2lkdGgiOjQwLjM0NTMxNDAyNTg3ODkwNiwiaGVpZ2h0IjoxLjQ1MDU0MjgwNzU3OTA0MDV9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ni4zMjY1MTkwMTI0NTExNywibGVmdCI6OS4wNDc2MDE2OTk4MjkxMDIsIndpZHRoIjoyNy45Njc0NDcyODA4ODM3OSwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ny43MDAzODIyMzI2NjYwMTYsImxlZnQiOjEyLjM4MDg3ODQ0ODQ4NjMyOCwid2lkdGgiOjM3LjAzNDc4NjIyNDM2NTIzNCwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0OS4wNzQyNDU0NTI4ODA4NiwibGVmdCI6OS4wNDc2MDE2OTk4MjkxMDIsIndpZHRoIjo0MC4zNDA2NDQ4MzY0MjU3OCwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1MC4zNTc1ODIwOTIyODUxNTYsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzQ0ODUyNDQ3NTA5NzY2LCJoZWlnaHQiOjEuNDUwNTQyODA3NTc5MDQwNX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEzMSJ9fV0%3D. These approaches counter traditional inefficiencies—evaporation, overuse—and support sustainability amid climate pressures (Daraz et al., 2025)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlVtYXIiLCJmYW1pbHkiOiJEYXJheiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiLFoHRlZmFuIiwiZmFtaWx5IjoiQm9qbmVjIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllvdW5hcyIsImZhbWlseSI6IktoYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMDc3LTA0NzIiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzOTAvYWdyaWN1bHR1cmUxNTA1MDU1NCIsIm9wZW5hbGV4IjoiVzQ0MDgyMTk4MTEiLCJleGEiOiJodHRwczovL3d3dy5tZHBpLmNvbS8yMDc3LTA0NzIvMTUvNS81NTQifSwidGl0bGUiOiJFbmVyZ3ktRWZmaWNpZW50IFNtYXJ0IElycmlnYXRpb24gVGVjaG5vbG9naWVzOiBBIFBhdGh3YXkgdG8gV2F0ZXIgYW5kIEVuZXJneSBTdXN0YWluYWJpbGl0eSBpbiBBZ3JpY3VsdHVyZSIsImFic3RyYWN0IjoiVGhlIGFncmljdWx0dXJhbCBzZWN0b3IgZmFjZXMgY2hhbGxlbmdlcyBzdWNoIGFzIHdhdGVyIHNjYXJjaXR5LCBlbmVyZ3kgaW5lZmZpY2llbmN5LCBhbmQgZGVjbGluaW5nIHByb2R1Y3Rpdml0eSwgcGFydGljdWxhcmx5IGluIGFyaWQgcmVnaW9ucy4gVHJhZGl0aW9uYWwgaXJyaWdhdGlvbiBtZXRob2RzIGNvbnRyaWJ1dGUgdG8gcmVzb3VyY2UgZGVwbGV0aW9uIGFuZCBlbnZpcm9ubWVudGFsIGltcGFjdHMuIFNvbGFyLXBvd2VyZWQgc21hcnQgaXJyaWdhdGlvbiBzeXN0ZW1zIGludGVncmF0ZSBwcmVjaXNpb24gaXJyaWdhdGlvbiB3aXRoIHJlbmV3YWJsZSBlbmVyZ3ksIGltcHJvdmluZyB3YXRlciB1c2UgYW5kIHByb2R1Y3Rpdml0eS4gSW4gUGFraXN0YW4sIHdoZXJlIGFncmljdWx0dXJlIGNvbnRyaWJ1dGVzIDE5JSBvZiBncm9zcyBkb21lc3RpYyBwcm9kdWN0IGFuZCBlbXBsb3lzIDQwJSBvZiB0aGUgd29ya2ZvcmNlLCB0aGVzZSBjaGFsbGVuZ2VzIGFyZSBzZXZlcmUsIGVzcGVjaWFsbHkgaW4gd2F0ZXItc2NhcmNlIGFyZWFzIGxpa2UgdGhlIENob2xpc3RhbiBEZXNlcnQuIFRoaXMgc3R1ZHkgZXhhbWluZXMgdGhlIGltcGFjdCBvZiBzb2xhci1wb3dlcmVkIHNtYXJ0IGlycmlnYXRpb24gb24gYWdyaWN1bHR1cmFsIHByb2R1Y3Rpdml0eSwgd2F0ZXIgY29uc2VydmF0aW9uLCBhbmQgZW5lcmd5IGVmZmljaWVuY3kgaW4gdGhlIENob2xpc3RhbiBEZXNlcnQuIFVzaW5nIGEgcXVhbnRpdGF0aXZlIGNyb3NzLXNlY3Rpb25hbCBkZXNpZ24sIGRhdGEgd2VyZSBjb2xsZWN0ZWQgZnJvbSAzODQgZmFybWVycyB2aWEgc3RydWN0dXJlZCBxdWVzdGlvbm5haXJlcy4gU3RhdGlzdGljYWwgYW5hbHlzZXMsIGluY2x1ZGluZyBtdWx0aXBsZSBsaW5lYXIgcmVncmVzc2lvbiwgcGFpcmVkIHNhbXBsZSB0LXRlc3RzLCBhbmQgU3RydWN0dXJhbCBFcXVhdGlvbiBNb2RlbGluZyAoU0VNKSwgd2VyZSBjb25kdWN0ZWQuIEZpbmRpbmdzIHNob3cgc2lnbmlmaWNhbnQgaW1wcm92ZW1lbnRzIGluIGNyb3AgeWllbGQgKGZyb20gMy4wIHRvIDQuOCB0b25zL2hlY3RhcmUpIGFuZCByZWR1Y3Rpb25zIGluIHdhdGVyIGFuZCBlbmVyZ3kgY29uc3VtcHRpb24uIFJlZ3Jlc3Npb24gYW5hbHlzaXMgaGlnaGxpZ2h0ZWQgc3Ryb25nIHBvc2l0aXZlIGVmZmVjdHMgb24geWllbGQgYW5kIGVmZmljaWVuY3ksIHdoaWxlIFNFTSBjb25maXJtZWQgcmVkdWNlZCBlbnZpcm9ubWVudGFsIGltcGFjdCBhbmQgb3BlcmF0aW9uYWwgY29zdHMuIFRoZSBzdHVkeSBjb25jbHVkZXMgdGhhdCBzb2xhci1wb3dlcmVkIGlycmlnYXRpb24gZW5oYW5jZXMgcHJvZHVjdGl2aXR5LCBjb25zZXJ2ZXMgcmVzb3VyY2VzLCBhbmQgcHJvbW90ZXMgc3VzdGFpbmFiaWxpdHkuIFBvbGljeW1ha2VycyBzaG91bGQgcHJvdmlkZSBmaW5hbmNpYWwgaW5jZW50aXZlcywgaW52ZXN0IGluIHJlbmV3YWJsZSBpbmZyYXN0cnVjdHVyZSwgYW5kIGltcGxlbWVudCB0cmFpbmluZyBwcm9ncmFtcyB0byBzdXBwb3J0IGFkb3B0aW9uLiBDb2xsYWJvcmF0aXZlIGVmZm9ydHMgYXJlIGVzc2VudGlhbCBmb3Igc3VzdGFpbmFibGUgYWdyaWN1bHR1cmUgaW4gYXJpZCByZWdpb25zLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJpc3N1ZWREYXRlIjoiMjAyNS0wMy0wNSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkFncmljdWx0dXJlIiwiaXNzdWUiOjUsInZvbHVtZSI6MTUsImltcGFjdEZhY3RvciI6IjUuMjYifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjo1NTQsImxhc3RQYWdlIjo1NTR9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiTXVsdGlkaXNjaXBsaW5hcnkgRGlnaXRhbCBQdWJsaXNoaW5nIEluc3RpdHV0ZSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NDgsImNpdGVkQnlDb3VudCI6Mn19LCJxdW90ZSI6IjEuIEludHJvZHVjdGlvbiBUaGUgYWdyaWN1bHR1cmFsIHNlY3RvciwgYSBjb3JuZXJzdG9uZSBvZiBnbG9iYWwgZm9vZCBzZWN1cml0eSBhbmQgZWNvbm9taWMgZGV2ZWxvcG1lbnQsIGZhY2VzIG1vdW50aW5nIGNoYWxsZW5nZXMgaW4gdGhlIDIxc3QgY2VudHVyeS4gQ2xpbWF0ZSBjaGFuZ2UsIHJhcGlkIHBvcHVsYXRpb24gZ3Jvd3RoLCBhbmQgZHdpbmRsaW5nIG5hdHVyYWwgcmVzb3VyY2VzIGhhdmUgaW50ZW5zaWZpZWQgdGhlIG5lZWQgZm9yIHN1c3RhaW5hYmxlIGFncmljdWx0dXJhbCBwcmFjdGljZXMuIEFtb25nIHRoZXNlIGNoYWxsZW5nZXMsIHdhdGVyIHNjYXJjaXR5IGFuZCBlbmVyZ3kgaW5lZmZpY2llbmN5IHN0YW5kIG91dCBhcyBjcml0aWNhbCBiYXJyaWVycyB0byBwcm9kdWN0aXZpdHkgYW5kIHN1c3RhaW5hYmlsaXR5LiBHbG9iYWxseSwgYWdyaWN1bHR1cmUgYWNjb3VudHMgZm9yIGFwcHJveGltYXRlbHkgNzAlIG9mIGZyZXNod2F0ZXIgd2l0aGRyYXdhbHMgYW5kIGlzIG9uZSBvZiB0aGUgbGFyZ2VzdCBjb25zdW1lcnMgb2YgZW5lcmd5IGluIG1hbnkgcmVnaW9ucyBbIDEsIDJdLiBUcmFkaXRpb25hbCBpcnJpZ2F0aW9uIHByYWN0aWNlcywgb2Z0ZW4gaW5lZmZpY2llbnQgYW5kIHJlc291cmNlLWludGVuc2l2ZSwgZXhhY2VyYmF0ZSB0aGVzZSBpc3N1ZXMsIGxlYWRpbmcgdG8gd2F0ZXIgd2FzdGFnZSwgZW5lcmd5IG92ZXJ1c2UsIGFuZCByZWR1Y2VkIGNyb3AgeWllbGRzLiJ9XQ%3D%3D.

### Sensor-Based Irrigation Scheduling

Sensor-based scheduling employs soil moisture probes (tensiometers, capacitance), weather stations, and plant stress sensors to trigger irrigation only when needed, preventing over- or under-watering (Jumintono et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikp1bWludG9ubyIsImZhbWlseSI6Ikp1bWludG9ubyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJOdXJuYXdhdHkiLCJmYW1pbHkiOiJOdXJuYXdhdHkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTnVncm9obyBUcmlzbnUiLCJmYW1pbHkiOiJCcmF0YSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJEaW5pIiwiZmFtaWx5IjoiUmFobWlhdGkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSGFkaSIsImZhbWlseSI6IlN1c2lsbyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBLiBBLiIsImZhbWlseSI6Ikh1c3NlbiJ9fV0sImlkcyI6eyJpc3NuIjpbIjE0MjktNzQyNiIsIjIwODMtNDUzNSJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMjQ0MjUvandsZC4yMDI0LjE1MTc5OCIsIm9wZW5hbGV4IjoiVzQ0MDU0NzcxMDAifSwidGl0bGUiOiJDb21wYXJhdGl2ZSBhbmFseXNpcyBvZiBwcmVjaXNpb24sIGRlZmljaXQsIGFuZCBkcmlwIGlycmlnYXRpb24gZm9yIHdhdGVyIGVmZmljaWVuY3kgaW4gSXJhcSIsImFic3RyYWN0IjoiU3VzdGFpbmFibGUgaXJyaWdhdGlvbiBwcmFjdGljZXMgYXJlIGNyaXRpY2FsIGZvciBhZGRyZXNzaW5nIHRoZSBjaGFsbGVuZ2VzIG9mIHdhdGVyIHNjYXJjaXR5IGFuZCBmb29kIGluc2VjdXJpdHkgaW4gdGhlIGZhY2Ugb2YgY2xpbWF0ZSBjaGFuZ2UgYW5kIGdyb3dpbmcgZ2xvYmFsIGZvb2QgZGVtYW5kLiBUaGlzIHN0dWR5IGludmVzdGlnYXRlZCB0aGUgcG90ZW50aWFsIG9mIHByZWNpc2lvbiBpcnJpZ2F0aW9uIChQSSksIGRlZmljaXQgaXJyaWdhdGlvbiAoREkpLCBhbmQgZHJpcCBpcnJpZ2F0aW9uIChEUkkpIHRlY2huaXF1ZXMgdG8gZW5oYW5jZSBjcm9wIHByb2R1Y3Rpdml0eSBhbmQgd2F0ZXIgdXNlIGVmZmljaWVuY3kgKFdVRSkgY29tcGFyZWQgdG8gY29udmVudGlvbmFsIGZsb29kIGlycmlnYXRpb24gKENGSSkgaW4gdGhyZWUgZGlzdGluY3QgYWdyb2Vjb2xvZ2ljYWwgem9uZXMgb2YgSXJhcS4gRmllbGQgZXhwZXJpbWVudHMgd2VyZSBjb25kdWN0ZWQgdXNpbmcgYSByYW5kb21pc2VkIGNvbXBsZXRlIGJsb2NrIGRlc2lnbiB3aXRoIHdoZWF0LCBtYWl6ZSwgYW5kIHJpY2UgY3JvcHMuIFRoZSByZXN1bHRzIGRlbW9uc3RyYXRlZCB0aGF0IFBJIHNpZ25pZmljYW50bHkgaW5jcmVhc2VkIGNyb3AgZ3Jvd3RoIHBhcmFtZXRlcnMsIGdyYWluIHlpZWxkLCBhbmQgV1VFIGFjcm9zcyBhbGwgem9uZXMsIHdpdGggeWllbGQgaW1wcm92ZW1lbnRzIG9mIDMz4oCTMzglIGFuZCBXVUUgaW5jcmVhc2VzIG9mIDQ24oCTNTElIGluIGNvbnRyYXN0IHdpdGggQ0ZJLiBUaGUgREkgYW5kIERSSSB0cmVhdG1lbnRzIGFsc28gb3V0cGVyZm9ybWVkIENGSSwgYWxiZWl0IHRvIGEgbGVzc2VyIGV4dGVudC4gUmVtb3RlIHNlbnNpbmctZGVyaXZlZCB2ZWdldGF0aW9uIGluZGljZXMgc3Ryb25nbHkgY29ycmVsYXRlZCB3aXRoIGNyb3AgZ3Jvd3RoIHBhcmFtZXRlcnMgYW5kIHlpZWxkLCB3aGlsZSBoeWRyb2xvZ2ljYWwgbW9kZWxsaW5nIHJldmVhbGVkIHJlZHVjZWQgZXZhcG90cmFuc3BpcmF0aW9uIGFuZCBzdXJmYWNlIHJ1bm9mZiB1bmRlciB0aGUgUEkgdHJlYXRtZW50LiBUaGUgc3VzdGFpbmFibGUgaXJyaWdhdGlvbiBwcmFjdGljZXMgcmVzdWx0ZWQgaW4gc3Vic3RhbnRpYWwgd2F0ZXIgc2F2aW5ncyBvZiAyMOKAkzMwJSBjb21wYXJlZCB0byBDRkkuIFRoZXNlIGZpbmRpbmdzIGhpZ2hsaWdodCB0aGUgaW1wb3J0YW5jZSBvZiBhZG9wdGluZyBlZmZpY2llbnQgaXJyaWdhdGlvbiB0ZWNobmlxdWVzLCBhbG9uZyB3aXRoIGEgaG9saXN0aWMgYXBwcm9hY2ggZW5jb21wYXNzaW5nIHRlY2hub2xvZ2ljYWwgaW5ub3ZhdGlvbnMsIGNhcGFjaXR5IGJ1aWxkaW5nLCBhbmQgc3Rha2Vob2xkZXIgZW5nYWdlbWVudCwgdG8gcHJvbW90ZSB3YXRlci0gZWZmaWNpZW50IGFncmljdWx0dXJlIGFuZCBlbnN1cmUgZm9vZCBzZWN1cml0eSBpbiB3YXRlci1zY2FyY2UgcmVnaW9ucy4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjQ0MjUvandsZC4yMDI0LjE1MTc5OCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjI0NDI1L2p3bGQuMjAyNC4xNTE3OTgiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMi0xNCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkpvdXJuYWwgb2YgV2F0ZXIgYW5kIExhbmQgRGV2ZWxvcG1lbnQiLCJpbXBhY3RGYWN0b3IiOiIxLjQ2In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTMwLCJsYXN0UGFnZSI6MTM2fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkRlIEdydXl0ZXIgT3BlbiJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MzIsImNpdGVkQnlDb3VudCI6MX19LCJxdW90ZSI6IkluIHRoaXMgY29udGV4dCwgdGhlIGFkb3B0aW9uIG9mICBzdXN0YWluYWJsZSBpcnJpZ2F0aW9uIHByYWN0aWNlcyBoYXMgZW1lcmdlZCBhcyBhIGNyaXRpY2FsIHN0cmF0ZWd5ICBmb3IgZW5oYW5jaW5nIGZvb2Qgc2VjdXJpdHkgd2hpbGUgcHJvbW90aW5nIHdhdGVyIGNvbnNlcnZhdGlvbiAgaW4gYWdyaWN1bHR1cmFsIGxhbmRzY2FwZXMuICBJcnJpZ2F0aW9uIHBsYXlzIGEgdml0YWwgcm9sZSBpbiBnbG9iYWwgZm9vZCBwcm9kdWN0aW9uLCB3aXRoICBpcnJpZ2F0ZWQgYWdyaWN1bHR1cmUgYWNjb3VudGluZyBmb3IgYXBwcm94aW1hdGVseSA0MCUgb2YgdGhlICBmb29kIHN1cHBseSAoV3JhY2hpZW4gZGUsIFNjaHVsdHogYW5kIEdvbGksIDIwMjE7IFNjaG1pdHQsIFJvc2EgIGFuZCBEYWlseSwgMjAyMikuIEhvd2V2ZXIsIHRyYWRpdGlvbmFsIGlycmlnYXRpb24gbWV0aG9kcywgc3VjaCBhcyAgZmxvb2QgYW5kIGZ1cnJvdyBpcnJpZ2F0aW9uLCBhcmUgb2Z0ZW4gaW5lZmZpY2llbnQsIGxlYWRpbmcgdG8gIHN1YnN0YW50aWFsIHdhdGVyIGxvc3NlcyB0aHJvdWdoIGV2YXBvcmF0aW9uLCBydW5vZmYsIGFuZCBkZWVwICBwZXJjb2xhdGlvbiAoQW1ib21zYSwgMjAyMCkuIE1vcmVvdmVyLCB0aGUgZXhjZXNzaXZlIGFwcGxpY2EtdGlvbiBvZiB3YXRlciBjYW4gcmVzdWx0IGluIHNvaWwgc2FsaW5pc2F0aW9uLCBudXRyaWVudCBsZWFjaGluZywgYW5kICBncm91bmR3YXRlciBkZXBsZXRpb24sIGZ1cnRoZXIgY29tcHJvbWlzaW5nIHRoZSBsb25nLXRlcm0gIHN1c3RhaW5hYmlsaXR5IG9mIGFncmljdWx0dXJhbCBzeXN0ZW1zIChZaW4gZXQgYWwuLCAyMDIyKS4gVG8gYWRkcmVzcyAgdGhlc2UgY2hhbGxlbmdlcywgcmVzZWFyY2hlcnMgYW5kIHByYWN0aXRpb25lcnMgaGF2ZSBiZWVuIGV4cGxvci1pbmcgaW5ub3ZhdGl2ZSBpcnJpZ2F0aW9uIHRlY2huaXF1ZXMgYW5kIHdhdGVyIG1hbmFnZW1lbnQgIHN0cmF0ZWdpZXMgdGhhdCBjYW4gb3B0aW1pc2Ugd2F0ZXIgdXNlIGVmZmljaWVuY3kgKFdVRSkgd2hpbGUgIG1haW50YWluaW5nIG9yIGVuaGFuY2luZyBjcm9wIHByb2R1Y3Rpdml0eS4gIFByZWNpc2lvbiBpcnJpZ2F0aW9uIChQSSkgb2ZmZXJzIGEgcHJvbWlzaW5nIHNvbHV0aW9uLCAgaW52b2x2aW5nIHRoZSB0YXJnZXRlZCBhcHBsaWNhdGlvbiBvZiB3YXRlciBzcGVjaWZpY2FsbHkgdG8gdGhlICBuZWVkcyBvZiBpbmRpdmlkdWFsIHBsYW50cyBvciBjcm9wIHpvbmVzIChaaW5rZXJuYWdlbCBldCBhbC4sIDIwMjApLiAgIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyMi44ODY3MzU5MTYxMzc2OTUsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzQ2MjIxOTIzODI4MTI1LCJoZWlnaHQiOjEuNDUwNTQxMDE5NDM5Njk3M319LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI0LjM1MTEyMzgwOTgxNDQ1MywibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTI3MTYwNjQ0NSwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI1LjcyNDk4NzAzMDAyOTI5NywibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTY1MzA3NjE3MiwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI3LjA5ODg1MDI1MDI0NDE0LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjE2LjQxODE2MTM5MjIxMTkxNCwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI4LjQ3MjcwOTY1NTc2MTcyLCJsZWZ0IjoxMi4zODA4ODAzNTU4MzQ5NjEsIndpZHRoIjozNy4wMTIxODQxNDMwNjY0MDYsImhlaWdodCI6MS4zNjAwMTQ3OTYyNTcwMTl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyOS44NDY1NzI4NzU5NzY1NjIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQwNjQ4NjUxMTIzMDUsImhlaWdodCI6MS4zNjAwMTY3MDM2MDU2NTE5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzEuMjE0MDM2OTQxNTI4MzIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQ0ODU2MjYyMjA3MDMsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzIuNTg3OTAyMDY5MDkxOCwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDYwNTc4OTE4NDU3LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMzLjk2MTc2MTQ3NDYwOTM3NSwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDA0ODQ2MTkxNDA2MjUsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzUuMzM1NjI0Njk0ODI0MjIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzM5NzQ0NTY3ODcxMDk0LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjM2LjcwOTQ4NzkxNTAzOTA2LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjAwMTU5MDcyODc1OTc2NiwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC4wODMzNDczMjA1NTY2NCwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTY1MzA3NjE3MiwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOS40NTcyMTA1NDA3NzE0ODQsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQzMDQ4MDk1NzAzMTI1LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQwLjc0MDU0NzE4MDE3NTc4LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjM0NTQ1NTE2OTY3NzczNCwiaGVpZ2h0IjoxLjQ1MDU0MjgwNzU3OTA0MDV9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Mi4yMDQ5MzMxNjY1MDM5MDYsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMDAzMjUwMTIyMDcwMzEsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDMuNTc4Nzk2Mzg2NzE4NzUsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzM3OTIxMTQyNTc4MTI1LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQ0Ljg2MjEzMzAyNjEyMzA1LCJsZWZ0Ijo5LjA0NzYwMTY5OTgyOTEwMiwid2lkdGgiOjQwLjM0NTMxNDAyNTg3ODkwNiwiaGVpZ2h0IjoxLjQ1MDU0MjgwNzU3OTA0MDV9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ni4zMjY1MTkwMTI0NTExNywibGVmdCI6OS4wNDc2MDE2OTk4MjkxMDIsIndpZHRoIjoyNy45Njc0NDcyODA4ODM3OSwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ny43MDAzODIyMzI2NjYwMTYsImxlZnQiOjEyLjM4MDg3ODQ0ODQ4NjMyOCwid2lkdGgiOjM3LjAzNDc4NjIyNDM2NTIzNCwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0OS4wNzQyNDU0NTI4ODA4NiwibGVmdCI6OS4wNDc2MDE2OTk4MjkxMDIsIndpZHRoIjo0MC4zNDA2NDQ4MzY0MjU3OCwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1MC4zNTc1ODIwOTIyODUxNTYsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzQ0ODUyNDQ3NTA5NzY2LCJoZWlnaHQiOjEuNDUwNTQyODA3NTc5MDQwNX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEzMSJ9fV0%3D.

### Table 5: Water Management Strategies for Sustainability

| Strategy | Primary Objective | Environmental Impact |
| --- | --- | --- |
| Rainwater Harvesting | Collecting runoff | Reduces groundwater reliance |
| Wastewater Recycling | Non-potable reuse | Minimizes freshwater withdrawal |
| Desalination | Saline water treatment | New water source in arid areas |
| Deficit Irrigation | Managed water stress | High water use efficiency |
| Drought-Resistant Crops | Genetic adaptation | Resilience to climate variability |
| Soil Conservation | Moisture retention | Prevents land degradation |
| Smart Scheduling | Demand-based delivery | Curbing water wastage |
| Precision Delivery | Individual plant targeting | Up to 90% wastage reduction |

Principles of Operation

Sensors measure parameters like volumetric water content, evapotranspiration, or leaf water potential, feeding data to controllers via wireless networks for automated valve adjustments. Threshold-based algorithms (e.g., 50% depletion) optimize timing (Daraz et al., 2025)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlVtYXIiLCJmYW1pbHkiOiJEYXJheiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiLFoHRlZmFuIiwiZmFtaWx5IjoiQm9qbmVjIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllvdW5hcyIsImZhbWlseSI6IktoYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMDc3LTA0NzIiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzOTAvYWdyaWN1bHR1cmUxNTA1MDU1NCIsIm9wZW5hbGV4IjoiVzQ0MDgyMTk4MTEiLCJleGEiOiJodHRwczovL3d3dy5tZHBpLmNvbS8yMDc3LTA0NzIvMTUvNS81NTQifSwidGl0bGUiOiJFbmVyZ3ktRWZmaWNpZW50IFNtYXJ0IElycmlnYXRpb24gVGVjaG5vbG9naWVzOiBBIFBhdGh3YXkgdG8gV2F0ZXIgYW5kIEVuZXJneSBTdXN0YWluYWJpbGl0eSBpbiBBZ3JpY3VsdHVyZSIsImFic3RyYWN0IjoiVGhlIGFncmljdWx0dXJhbCBzZWN0b3IgZmFjZXMgY2hhbGxlbmdlcyBzdWNoIGFzIHdhdGVyIHNjYXJjaXR5LCBlbmVyZ3kgaW5lZmZpY2llbmN5LCBhbmQgZGVjbGluaW5nIHByb2R1Y3Rpdml0eSwgcGFydGljdWxhcmx5IGluIGFyaWQgcmVnaW9ucy4gVHJhZGl0aW9uYWwgaXJyaWdhdGlvbiBtZXRob2RzIGNvbnRyaWJ1dGUgdG8gcmVzb3VyY2UgZGVwbGV0aW9uIGFuZCBlbnZpcm9ubWVudGFsIGltcGFjdHMuIFNvbGFyLXBvd2VyZWQgc21hcnQgaXJyaWdhdGlvbiBzeXN0ZW1zIGludGVncmF0ZSBwcmVjaXNpb24gaXJyaWdhdGlvbiB3aXRoIHJlbmV3YWJsZSBlbmVyZ3ksIGltcHJvdmluZyB3YXRlciB1c2UgYW5kIHByb2R1Y3Rpdml0eS4gSW4gUGFraXN0YW4sIHdoZXJlIGFncmljdWx0dXJlIGNvbnRyaWJ1dGVzIDE5JSBvZiBncm9zcyBkb21lc3RpYyBwcm9kdWN0IGFuZCBlbXBsb3lzIDQwJSBvZiB0aGUgd29ya2ZvcmNlLCB0aGVzZSBjaGFsbGVuZ2VzIGFyZSBzZXZlcmUsIGVzcGVjaWFsbHkgaW4gd2F0ZXItc2NhcmNlIGFyZWFzIGxpa2UgdGhlIENob2xpc3RhbiBEZXNlcnQuIFRoaXMgc3R1ZHkgZXhhbWluZXMgdGhlIGltcGFjdCBvZiBzb2xhci1wb3dlcmVkIHNtYXJ0IGlycmlnYXRpb24gb24gYWdyaWN1bHR1cmFsIHByb2R1Y3Rpdml0eSwgd2F0ZXIgY29uc2VydmF0aW9uLCBhbmQgZW5lcmd5IGVmZmljaWVuY3kgaW4gdGhlIENob2xpc3RhbiBEZXNlcnQuIFVzaW5nIGEgcXVhbnRpdGF0aXZlIGNyb3NzLXNlY3Rpb25hbCBkZXNpZ24sIGRhdGEgd2VyZSBjb2xsZWN0ZWQgZnJvbSAzODQgZmFybWVycyB2aWEgc3RydWN0dXJlZCBxdWVzdGlvbm5haXJlcy4gU3RhdGlzdGljYWwgYW5hbHlzZXMsIGluY2x1ZGluZyBtdWx0aXBsZSBsaW5lYXIgcmVncmVzc2lvbiwgcGFpcmVkIHNhbXBsZSB0LXRlc3RzLCBhbmQgU3RydWN0dXJhbCBFcXVhdGlvbiBNb2RlbGluZyAoU0VNKSwgd2VyZSBjb25kdWN0ZWQuIEZpbmRpbmdzIHNob3cgc2lnbmlmaWNhbnQgaW1wcm92ZW1lbnRzIGluIGNyb3AgeWllbGQgKGZyb20gMy4wIHRvIDQuOCB0b25zL2hlY3RhcmUpIGFuZCByZWR1Y3Rpb25zIGluIHdhdGVyIGFuZCBlbmVyZ3kgY29uc3VtcHRpb24uIFJlZ3Jlc3Npb24gYW5hbHlzaXMgaGlnaGxpZ2h0ZWQgc3Ryb25nIHBvc2l0aXZlIGVmZmVjdHMgb24geWllbGQgYW5kIGVmZmljaWVuY3ksIHdoaWxlIFNFTSBjb25maXJtZWQgcmVkdWNlZCBlbnZpcm9ubWVudGFsIGltcGFjdCBhbmQgb3BlcmF0aW9uYWwgY29zdHMuIFRoZSBzdHVkeSBjb25jbHVkZXMgdGhhdCBzb2xhci1wb3dlcmVkIGlycmlnYXRpb24gZW5oYW5jZXMgcHJvZHVjdGl2aXR5LCBjb25zZXJ2ZXMgcmVzb3VyY2VzLCBhbmQgcHJvbW90ZXMgc3VzdGFpbmFiaWxpdHkuIFBvbGljeW1ha2VycyBzaG91bGQgcHJvdmlkZSBmaW5hbmNpYWwgaW5jZW50aXZlcywgaW52ZXN0IGluIHJlbmV3YWJsZSBpbmZyYXN0cnVjdHVyZSwgYW5kIGltcGxlbWVudCB0cmFpbmluZyBwcm9ncmFtcyB0byBzdXBwb3J0IGFkb3B0aW9uLiBDb2xsYWJvcmF0aXZlIGVmZm9ydHMgYXJlIGVzc2VudGlhbCBmb3Igc3VzdGFpbmFibGUgYWdyaWN1bHR1cmUgaW4gYXJpZCByZWdpb25zLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJpc3N1ZWREYXRlIjoiMjAyNS0wMy0wNSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkFncmljdWx0dXJlIiwiaXNzdWUiOjUsInZvbHVtZSI6MTUsImltcGFjdEZhY3RvciI6IjUuMjYifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjo1NTQsImxhc3RQYWdlIjo1NTR9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiTXVsdGlkaXNjaXBsaW5hcnkgRGlnaXRhbCBQdWJsaXNoaW5nIEluc3RpdHV0ZSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NDgsImNpdGVkQnlDb3VudCI6Mn19LCJxdW90ZSI6IjEuIEludHJvZHVjdGlvbiBUaGUgYWdyaWN1bHR1cmFsIHNlY3RvciwgYSBjb3JuZXJzdG9uZSBvZiBnbG9iYWwgZm9vZCBzZWN1cml0eSBhbmQgZWNvbm9taWMgZGV2ZWxvcG1lbnQsIGZhY2VzIG1vdW50aW5nIGNoYWxsZW5nZXMgaW4gdGhlIDIxc3QgY2VudHVyeS4gQ2xpbWF0ZSBjaGFuZ2UsIHJhcGlkIHBvcHVsYXRpb24gZ3Jvd3RoLCBhbmQgZHdpbmRsaW5nIG5hdHVyYWwgcmVzb3VyY2VzIGhhdmUgaW50ZW5zaWZpZWQgdGhlIG5lZWQgZm9yIHN1c3RhaW5hYmxlIGFncmljdWx0dXJhbCBwcmFjdGljZXMuIEFtb25nIHRoZXNlIGNoYWxsZW5nZXMsIHdhdGVyIHNjYXJjaXR5IGFuZCBlbmVyZ3kgaW5lZmZpY2llbmN5IHN0YW5kIG91dCBhcyBjcml0aWNhbCBiYXJyaWVycyB0byBwcm9kdWN0aXZpdHkgYW5kIHN1c3RhaW5hYmlsaXR5LiBHbG9iYWxseSwgYWdyaWN1bHR1cmUgYWNjb3VudHMgZm9yIGFwcHJveGltYXRlbHkgNzAlIG9mIGZyZXNod2F0ZXIgd2l0aGRyYXdhbHMgYW5kIGlzIG9uZSBvZiB0aGUgbGFyZ2VzdCBjb25zdW1lcnMgb2YgZW5lcmd5IGluIG1hbnkgcmVnaW9ucyBbIDEsIDJdLiBUcmFkaXRpb25hbCBpcnJpZ2F0aW9uIHByYWN0aWNlcywgb2Z0ZW4gaW5lZmZpY2llbnQgYW5kIHJlc291cmNlLWludGVuc2l2ZSwgZXhhY2VyYmF0ZSB0aGVzZSBpc3N1ZXMsIGxlYWRpbmcgdG8gd2F0ZXIgd2FzdGFnZSwgZW5lcmd5IG92ZXJ1c2UsIGFuZCByZWR1Y2VkIGNyb3AgeWllbGRzLiJ9XQ%3D%3D.

Advantages and Efficiency Gains

- Precision water use: 30-50% savings by matching crop ET demands (Daraz et al., 2025)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlVtYXIiLCJmYW1pbHkiOiJEYXJheiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiLFoHRlZmFuIiwiZmFtaWx5IjoiQm9qbmVjIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllvdW5hcyIsImZhbWlseSI6IktoYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMDc3LTA0NzIiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzOTAvYWdyaWN1bHR1cmUxNTA1MDU1NCIsIm9wZW5hbGV4IjoiVzQ0MDgyMTk4MTEiLCJleGEiOiJodHRwczovL3d3dy5tZHBpLmNvbS8yMDc3LTA0NzIvMTUvNS81NTQifSwidGl0bGUiOiJFbmVyZ3ktRWZmaWNpZW50IFNtYXJ0IElycmlnYXRpb24gVGVjaG5vbG9naWVzOiBBIFBhdGh3YXkgdG8gV2F0ZXIgYW5kIEVuZXJneSBTdXN0YWluYWJpbGl0eSBpbiBBZ3JpY3VsdHVyZSIsImFic3RyYWN0IjoiVGhlIGFncmljdWx0dXJhbCBzZWN0b3IgZmFjZXMgY2hhbGxlbmdlcyBzdWNoIGFzIHdhdGVyIHNjYXJjaXR5LCBlbmVyZ3kgaW5lZmZpY2llbmN5LCBhbmQgZGVjbGluaW5nIHByb2R1Y3Rpdml0eSwgcGFydGljdWxhcmx5IGluIGFyaWQgcmVnaW9ucy4gVHJhZGl0aW9uYWwgaXJyaWdhdGlvbiBtZXRob2RzIGNvbnRyaWJ1dGUgdG8gcmVzb3VyY2UgZGVwbGV0aW9uIGFuZCBlbnZpcm9ubWVudGFsIGltcGFjdHMuIFNvbGFyLXBvd2VyZWQgc21hcnQgaXJyaWdhdGlvbiBzeXN0ZW1zIGludGVncmF0ZSBwcmVjaXNpb24gaXJyaWdhdGlvbiB3aXRoIHJlbmV3YWJsZSBlbmVyZ3ksIGltcHJvdmluZyB3YXRlciB1c2UgYW5kIHByb2R1Y3Rpdml0eS4gSW4gUGFraXN0YW4sIHdoZXJlIGFncmljdWx0dXJlIGNvbnRyaWJ1dGVzIDE5JSBvZiBncm9zcyBkb21lc3RpYyBwcm9kdWN0IGFuZCBlbXBsb3lzIDQwJSBvZiB0aGUgd29ya2ZvcmNlLCB0aGVzZSBjaGFsbGVuZ2VzIGFyZSBzZXZlcmUsIGVzcGVjaWFsbHkgaW4gd2F0ZXItc2NhcmNlIGFyZWFzIGxpa2UgdGhlIENob2xpc3RhbiBEZXNlcnQuIFRoaXMgc3R1ZHkgZXhhbWluZXMgdGhlIGltcGFjdCBvZiBzb2xhci1wb3dlcmVkIHNtYXJ0IGlycmlnYXRpb24gb24gYWdyaWN1bHR1cmFsIHByb2R1Y3Rpdml0eSwgd2F0ZXIgY29uc2VydmF0aW9uLCBhbmQgZW5lcmd5IGVmZmljaWVuY3kgaW4gdGhlIENob2xpc3RhbiBEZXNlcnQuIFVzaW5nIGEgcXVhbnRpdGF0aXZlIGNyb3NzLXNlY3Rpb25hbCBkZXNpZ24sIGRhdGEgd2VyZSBjb2xsZWN0ZWQgZnJvbSAzODQgZmFybWVycyB2aWEgc3RydWN0dXJlZCBxdWVzdGlvbm5haXJlcy4gU3RhdGlzdGljYWwgYW5hbHlzZXMsIGluY2x1ZGluZyBtdWx0aXBsZSBsaW5lYXIgcmVncmVzc2lvbiwgcGFpcmVkIHNhbXBsZSB0LXRlc3RzLCBhbmQgU3RydWN0dXJhbCBFcXVhdGlvbiBNb2RlbGluZyAoU0VNKSwgd2VyZSBjb25kdWN0ZWQuIEZpbmRpbmdzIHNob3cgc2lnbmlmaWNhbnQgaW1wcm92ZW1lbnRzIGluIGNyb3AgeWllbGQgKGZyb20gMy4wIHRvIDQuOCB0b25zL2hlY3RhcmUpIGFuZCByZWR1Y3Rpb25zIGluIHdhdGVyIGFuZCBlbmVyZ3kgY29uc3VtcHRpb24uIFJlZ3Jlc3Npb24gYW5hbHlzaXMgaGlnaGxpZ2h0ZWQgc3Ryb25nIHBvc2l0aXZlIGVmZmVjdHMgb24geWllbGQgYW5kIGVmZmljaWVuY3ksIHdoaWxlIFNFTSBjb25maXJtZWQgcmVkdWNlZCBlbnZpcm9ubWVudGFsIGltcGFjdCBhbmQgb3BlcmF0aW9uYWwgY29zdHMuIFRoZSBzdHVkeSBjb25jbHVkZXMgdGhhdCBzb2xhci1wb3dlcmVkIGlycmlnYXRpb24gZW5oYW5jZXMgcHJvZHVjdGl2aXR5LCBjb25zZXJ2ZXMgcmVzb3VyY2VzLCBhbmQgcHJvbW90ZXMgc3VzdGFpbmFiaWxpdHkuIFBvbGljeW1ha2VycyBzaG91bGQgcHJvdmlkZSBmaW5hbmNpYWwgaW5jZW50aXZlcywgaW52ZXN0IGluIHJlbmV3YWJsZSBpbmZyYXN0cnVjdHVyZSwgYW5kIGltcGxlbWVudCB0cmFpbmluZyBwcm9ncmFtcyB0byBzdXBwb3J0IGFkb3B0aW9uLiBDb2xsYWJvcmF0aXZlIGVmZm9ydHMgYXJlIGVzc2VudGlhbCBmb3Igc3VzdGFpbmFibGUgYWdyaWN1bHR1cmUgaW4gYXJpZCByZWdpb25zLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJpc3N1ZWREYXRlIjoiMjAyNS0wMy0wNSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkFncmljdWx0dXJlIiwiaXNzdWUiOjUsInZvbHVtZSI6MTUsImltcGFjdEZhY3RvciI6IjUuMjYifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjo1NTQsImxhc3RQYWdlIjo1NTR9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiTXVsdGlkaXNjaXBsaW5hcnkgRGlnaXRhbCBQdWJsaXNoaW5nIEluc3RpdHV0ZSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NDgsImNpdGVkQnlDb3VudCI6Mn19LCJxdW90ZSI6IjEuIEludHJvZHVjdGlvbiBUaGUgYWdyaWN1bHR1cmFsIHNlY3RvciwgYSBjb3JuZXJzdG9uZSBvZiBnbG9iYWwgZm9vZCBzZWN1cml0eSBhbmQgZWNvbm9taWMgZGV2ZWxvcG1lbnQsIGZhY2VzIG1vdW50aW5nIGNoYWxsZW5nZXMgaW4gdGhlIDIxc3QgY2VudHVyeS4gQ2xpbWF0ZSBjaGFuZ2UsIHJhcGlkIHBvcHVsYXRpb24gZ3Jvd3RoLCBhbmQgZHdpbmRsaW5nIG5hdHVyYWwgcmVzb3VyY2VzIGhhdmUgaW50ZW5zaWZpZWQgdGhlIG5lZWQgZm9yIHN1c3RhaW5hYmxlIGFncmljdWx0dXJhbCBwcmFjdGljZXMuIEFtb25nIHRoZXNlIGNoYWxsZW5nZXMsIHdhdGVyIHNjYXJjaXR5IGFuZCBlbmVyZ3kgaW5lZmZpY2llbmN5IHN0YW5kIG91dCBhcyBjcml0aWNhbCBiYXJyaWVycyB0byBwcm9kdWN0aXZpdHkgYW5kIHN1c3RhaW5hYmlsaXR5LiBHbG9iYWxseSwgYWdyaWN1bHR1cmUgYWNjb3VudHMgZm9yIGFwcHJveGltYXRlbHkgNzAlIG9mIGZyZXNod2F0ZXIgd2l0aGRyYXdhbHMgYW5kIGlzIG9uZSBvZiB0aGUgbGFyZ2VzdCBjb25zdW1lcnMgb2YgZW5lcmd5IGluIG1hbnkgcmVnaW9ucyBbIDEsIDJdLiBUcmFkaXRpb25hbCBpcnJpZ2F0aW9uIHByYWN0aWNlcywgb2Z0ZW4gaW5lZmZpY2llbnQgYW5kIHJlc291cmNlLWludGVuc2l2ZSwgZXhhY2VyYmF0ZSB0aGVzZSBpc3N1ZXMsIGxlYWRpbmcgdG8gd2F0ZXIgd2FzdGFnZSwgZW5lcmd5IG92ZXJ1c2UsIGFuZCByZWR1Y2VkIGNyb3AgeWllbGRzLiJ9XQ%3D%3D;

- Energy reduction: Eliminates unnecessary pumping (Daraz et al., 2025)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlVtYXIiLCJmYW1pbHkiOiJEYXJheiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiLFoHRlZmFuIiwiZmFtaWx5IjoiQm9qbmVjIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllvdW5hcyIsImZhbWlseSI6IktoYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMDc3LTA0NzIiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzOTAvYWdyaWN1bHR1cmUxNTA1MDU1NCIsIm9wZW5hbGV4IjoiVzQ0MDgyMTk4MTEiLCJleGEiOiJodHRwczovL3d3dy5tZHBpLmNvbS8yMDc3LTA0NzIvMTUvNS81NTQifSwidGl0bGUiOiJFbmVyZ3ktRWZmaWNpZW50IFNtYXJ0IElycmlnYXRpb24gVGVjaG5vbG9naWVzOiBBIFBhdGh3YXkgdG8gV2F0ZXIgYW5kIEVuZXJneSBTdXN0YWluYWJpbGl0eSBpbiBBZ3JpY3VsdHVyZSIsImFic3RyYWN0IjoiVGhlIGFncmljdWx0dXJhbCBzZWN0b3IgZmFjZXMgY2hhbGxlbmdlcyBzdWNoIGFzIHdhdGVyIHNjYXJjaXR5LCBlbmVyZ3kgaW5lZmZpY2llbmN5LCBhbmQgZGVjbGluaW5nIHByb2R1Y3Rpdml0eSwgcGFydGljdWxhcmx5IGluIGFyaWQgcmVnaW9ucy4gVHJhZGl0aW9uYWwgaXJyaWdhdGlvbiBtZXRob2RzIGNvbnRyaWJ1dGUgdG8gcmVzb3VyY2UgZGVwbGV0aW9uIGFuZCBlbnZpcm9ubWVudGFsIGltcGFjdHMuIFNvbGFyLXBvd2VyZWQgc21hcnQgaXJyaWdhdGlvbiBzeXN0ZW1zIGludGVncmF0ZSBwcmVjaXNpb24gaXJyaWdhdGlvbiB3aXRoIHJlbmV3YWJsZSBlbmVyZ3ksIGltcHJvdmluZyB3YXRlciB1c2UgYW5kIHByb2R1Y3Rpdml0eS4gSW4gUGFraXN0YW4sIHdoZXJlIGFncmljdWx0dXJlIGNvbnRyaWJ1dGVzIDE5JSBvZiBncm9zcyBkb21lc3RpYyBwcm9kdWN0IGFuZCBlbXBsb3lzIDQwJSBvZiB0aGUgd29ya2ZvcmNlLCB0aGVzZSBjaGFsbGVuZ2VzIGFyZSBzZXZlcmUsIGVzcGVjaWFsbHkgaW4gd2F0ZXItc2NhcmNlIGFyZWFzIGxpa2UgdGhlIENob2xpc3RhbiBEZXNlcnQuIFRoaXMgc3R1ZHkgZXhhbWluZXMgdGhlIGltcGFjdCBvZiBzb2xhci1wb3dlcmVkIHNtYXJ0IGlycmlnYXRpb24gb24gYWdyaWN1bHR1cmFsIHByb2R1Y3Rpdml0eSwgd2F0ZXIgY29uc2VydmF0aW9uLCBhbmQgZW5lcmd5IGVmZmljaWVuY3kgaW4gdGhlIENob2xpc3RhbiBEZXNlcnQuIFVzaW5nIGEgcXVhbnRpdGF0aXZlIGNyb3NzLXNlY3Rpb25hbCBkZXNpZ24sIGRhdGEgd2VyZSBjb2xsZWN0ZWQgZnJvbSAzODQgZmFybWVycyB2aWEgc3RydWN0dXJlZCBxdWVzdGlvbm5haXJlcy4gU3RhdGlzdGljYWwgYW5hbHlzZXMsIGluY2x1ZGluZyBtdWx0aXBsZSBsaW5lYXIgcmVncmVzc2lvbiwgcGFpcmVkIHNhbXBsZSB0LXRlc3RzLCBhbmQgU3RydWN0dXJhbCBFcXVhdGlvbiBNb2RlbGluZyAoU0VNKSwgd2VyZSBjb25kdWN0ZWQuIEZpbmRpbmdzIHNob3cgc2lnbmlmaWNhbnQgaW1wcm92ZW1lbnRzIGluIGNyb3AgeWllbGQgKGZyb20gMy4wIHRvIDQuOCB0b25zL2hlY3RhcmUpIGFuZCByZWR1Y3Rpb25zIGluIHdhdGVyIGFuZCBlbmVyZ3kgY29uc3VtcHRpb24uIFJlZ3Jlc3Npb24gYW5hbHlzaXMgaGlnaGxpZ2h0ZWQgc3Ryb25nIHBvc2l0aXZlIGVmZmVjdHMgb24geWllbGQgYW5kIGVmZmljaWVuY3ksIHdoaWxlIFNFTSBjb25maXJtZWQgcmVkdWNlZCBlbnZpcm9ubWVudGFsIGltcGFjdCBhbmQgb3BlcmF0aW9uYWwgY29zdHMuIFRoZSBzdHVkeSBjb25jbHVkZXMgdGhhdCBzb2xhci1wb3dlcmVkIGlycmlnYXRpb24gZW5oYW5jZXMgcHJvZHVjdGl2aXR5LCBjb25zZXJ2ZXMgcmVzb3VyY2VzLCBhbmQgcHJvbW90ZXMgc3VzdGFpbmFiaWxpdHkuIFBvbGljeW1ha2VycyBzaG91bGQgcHJvdmlkZSBmaW5hbmNpYWwgaW5jZW50aXZlcywgaW52ZXN0IGluIHJlbmV3YWJsZSBpbmZyYXN0cnVjdHVyZSwgYW5kIGltcGxlbWVudCB0cmFpbmluZyBwcm9ncmFtcyB0byBzdXBwb3J0IGFkb3B0aW9uLiBDb2xsYWJvcmF0aXZlIGVmZm9ydHMgYXJlIGVzc2VudGlhbCBmb3Igc3VzdGFpbmFibGUgYWdyaWN1bHR1cmUgaW4gYXJpZCByZWdpb25zLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJpc3N1ZWREYXRlIjoiMjAyNS0wMy0wNSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkFncmljdWx0dXJlIiwiaXNzdWUiOjUsInZvbHVtZSI6MTUsImltcGFjdEZhY3RvciI6IjUuMjYifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjo1NTQsImxhc3RQYWdlIjo1NTR9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiTXVsdGlkaXNjaXBsaW5hcnkgRGlnaXRhbCBQdWJsaXNoaW5nIEluc3RpdHV0ZSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NDgsImNpdGVkQnlDb3VudCI6Mn19LCJxdW90ZSI6IjEuIEludHJvZHVjdGlvbiBUaGUgYWdyaWN1bHR1cmFsIHNlY3RvciwgYSBjb3JuZXJzdG9uZSBvZiBnbG9iYWwgZm9vZCBzZWN1cml0eSBhbmQgZWNvbm9taWMgZGV2ZWxvcG1lbnQsIGZhY2VzIG1vdW50aW5nIGNoYWxsZW5nZXMgaW4gdGhlIDIxc3QgY2VudHVyeS4gQ2xpbWF0ZSBjaGFuZ2UsIHJhcGlkIHBvcHVsYXRpb24gZ3Jvd3RoLCBhbmQgZHdpbmRsaW5nIG5hdHVyYWwgcmVzb3VyY2VzIGhhdmUgaW50ZW5zaWZpZWQgdGhlIG5lZWQgZm9yIHN1c3RhaW5hYmxlIGFncmljdWx0dXJhbCBwcmFjdGljZXMuIEFtb25nIHRoZXNlIGNoYWxsZW5nZXMsIHdhdGVyIHNjYXJjaXR5IGFuZCBlbmVyZ3kgaW5lZmZpY2llbmN5IHN0YW5kIG91dCBhcyBjcml0aWNhbCBiYXJyaWVycyB0byBwcm9kdWN0aXZpdHkgYW5kIHN1c3RhaW5hYmlsaXR5LiBHbG9iYWxseSwgYWdyaWN1bHR1cmUgYWNjb3VudHMgZm9yIGFwcHJveGltYXRlbHkgNzAlIG9mIGZyZXNod2F0ZXIgd2l0aGRyYXdhbHMgYW5kIGlzIG9uZSBvZiB0aGUgbGFyZ2VzdCBjb25zdW1lcnMgb2YgZW5lcmd5IGluIG1hbnkgcmVnaW9ucyBbIDEsIDJdLiBUcmFkaXRpb25hbCBpcnJpZ2F0aW9uIHByYWN0aWNlcywgb2Z0ZW4gaW5lZmZpY2llbnQgYW5kIHJlc291cmNlLWludGVuc2l2ZSwgZXhhY2VyYmF0ZSB0aGVzZSBpc3N1ZXMsIGxlYWRpbmcgdG8gd2F0ZXIgd2FzdGFnZSwgZW5lcmd5IG92ZXJ1c2UsIGFuZCByZWR1Y2VkIGNyb3AgeWllbGRzLiJ9XQ%3D%3D;

- Yield optimization: Maintains ideal soil conditions, enhancing quality (Jumintono et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikp1bWludG9ubyIsImZhbWlseSI6Ikp1bWludG9ubyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJOdXJuYXdhdHkiLCJmYW1pbHkiOiJOdXJuYXdhdHkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTnVncm9obyBUcmlzbnUiLCJmYW1pbHkiOiJCcmF0YSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJEaW5pIiwiZmFtaWx5IjoiUmFobWlhdGkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSGFkaSIsImZhbWlseSI6IlN1c2lsbyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBLiBBLiIsImZhbWlseSI6Ikh1c3NlbiJ9fV0sImlkcyI6eyJpc3NuIjpbIjE0MjktNzQyNiIsIjIwODMtNDUzNSJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMjQ0MjUvandsZC4yMDI0LjE1MTc5OCIsIm9wZW5hbGV4IjoiVzQ0MDU0NzcxMDAifSwidGl0bGUiOiJDb21wYXJhdGl2ZSBhbmFseXNpcyBvZiBwcmVjaXNpb24sIGRlZmljaXQsIGFuZCBkcmlwIGlycmlnYXRpb24gZm9yIHdhdGVyIGVmZmljaWVuY3kgaW4gSXJhcSIsImFic3RyYWN0IjoiU3VzdGFpbmFibGUgaXJyaWdhdGlvbiBwcmFjdGljZXMgYXJlIGNyaXRpY2FsIGZvciBhZGRyZXNzaW5nIHRoZSBjaGFsbGVuZ2VzIG9mIHdhdGVyIHNjYXJjaXR5IGFuZCBmb29kIGluc2VjdXJpdHkgaW4gdGhlIGZhY2Ugb2YgY2xpbWF0ZSBjaGFuZ2UgYW5kIGdyb3dpbmcgZ2xvYmFsIGZvb2QgZGVtYW5kLiBUaGlzIHN0dWR5IGludmVzdGlnYXRlZCB0aGUgcG90ZW50aWFsIG9mIHByZWNpc2lvbiBpcnJpZ2F0aW9uIChQSSksIGRlZmljaXQgaXJyaWdhdGlvbiAoREkpLCBhbmQgZHJpcCBpcnJpZ2F0aW9uIChEUkkpIHRlY2huaXF1ZXMgdG8gZW5oYW5jZSBjcm9wIHByb2R1Y3Rpdml0eSBhbmQgd2F0ZXIgdXNlIGVmZmljaWVuY3kgKFdVRSkgY29tcGFyZWQgdG8gY29udmVudGlvbmFsIGZsb29kIGlycmlnYXRpb24gKENGSSkgaW4gdGhyZWUgZGlzdGluY3QgYWdyb2Vjb2xvZ2ljYWwgem9uZXMgb2YgSXJhcS4gRmllbGQgZXhwZXJpbWVudHMgd2VyZSBjb25kdWN0ZWQgdXNpbmcgYSByYW5kb21pc2VkIGNvbXBsZXRlIGJsb2NrIGRlc2lnbiB3aXRoIHdoZWF0LCBtYWl6ZSwgYW5kIHJpY2UgY3JvcHMuIFRoZSByZXN1bHRzIGRlbW9uc3RyYXRlZCB0aGF0IFBJIHNpZ25pZmljYW50bHkgaW5jcmVhc2VkIGNyb3AgZ3Jvd3RoIHBhcmFtZXRlcnMsIGdyYWluIHlpZWxkLCBhbmQgV1VFIGFjcm9zcyBhbGwgem9uZXMsIHdpdGggeWllbGQgaW1wcm92ZW1lbnRzIG9mIDMz4oCTMzglIGFuZCBXVUUgaW5jcmVhc2VzIG9mIDQ24oCTNTElIGluIGNvbnRyYXN0IHdpdGggQ0ZJLiBUaGUgREkgYW5kIERSSSB0cmVhdG1lbnRzIGFsc28gb3V0cGVyZm9ybWVkIENGSSwgYWxiZWl0IHRvIGEgbGVzc2VyIGV4dGVudC4gUmVtb3RlIHNlbnNpbmctZGVyaXZlZCB2ZWdldGF0aW9uIGluZGljZXMgc3Ryb25nbHkgY29ycmVsYXRlZCB3aXRoIGNyb3AgZ3Jvd3RoIHBhcmFtZXRlcnMgYW5kIHlpZWxkLCB3aGlsZSBoeWRyb2xvZ2ljYWwgbW9kZWxsaW5nIHJldmVhbGVkIHJlZHVjZWQgZXZhcG90cmFuc3BpcmF0aW9uIGFuZCBzdXJmYWNlIHJ1bm9mZiB1bmRlciB0aGUgUEkgdHJlYXRtZW50LiBUaGUgc3VzdGFpbmFibGUgaXJyaWdhdGlvbiBwcmFjdGljZXMgcmVzdWx0ZWQgaW4gc3Vic3RhbnRpYWwgd2F0ZXIgc2F2aW5ncyBvZiAyMOKAkzMwJSBjb21wYXJlZCB0byBDRkkuIFRoZXNlIGZpbmRpbmdzIGhpZ2hsaWdodCB0aGUgaW1wb3J0YW5jZSBvZiBhZG9wdGluZyBlZmZpY2llbnQgaXJyaWdhdGlvbiB0ZWNobmlxdWVzLCBhbG9uZyB3aXRoIGEgaG9saXN0aWMgYXBwcm9hY2ggZW5jb21wYXNzaW5nIHRlY2hub2xvZ2ljYWwgaW5ub3ZhdGlvbnMsIGNhcGFjaXR5IGJ1aWxkaW5nLCBhbmQgc3Rha2Vob2xkZXIgZW5nYWdlbWVudCwgdG8gcHJvbW90ZSB3YXRlci0gZWZmaWNpZW50IGFncmljdWx0dXJlIGFuZCBlbnN1cmUgZm9vZCBzZWN1cml0eSBpbiB3YXRlci1zY2FyY2UgcmVnaW9ucy4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjQ0MjUvandsZC4yMDI0LjE1MTc5OCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjI0NDI1L2p3bGQuMjAyNC4xNTE3OTgiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMi0xNCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkpvdXJuYWwgb2YgV2F0ZXIgYW5kIExhbmQgRGV2ZWxvcG1lbnQiLCJpbXBhY3RGYWN0b3IiOiIxLjQ2In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTMwLCJsYXN0UGFnZSI6MTM2fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkRlIEdydXl0ZXIgT3BlbiJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MzIsImNpdGVkQnlDb3VudCI6MX19LCJxdW90ZSI6IkluIHRoaXMgY29udGV4dCwgdGhlIGFkb3B0aW9uIG9mICBzdXN0YWluYWJsZSBpcnJpZ2F0aW9uIHByYWN0aWNlcyBoYXMgZW1lcmdlZCBhcyBhIGNyaXRpY2FsIHN0cmF0ZWd5ICBmb3IgZW5oYW5jaW5nIGZvb2Qgc2VjdXJpdHkgd2hpbGUgcHJvbW90aW5nIHdhdGVyIGNvbnNlcnZhdGlvbiAgaW4gYWdyaWN1bHR1cmFsIGxhbmRzY2FwZXMuICBJcnJpZ2F0aW9uIHBsYXlzIGEgdml0YWwgcm9sZSBpbiBnbG9iYWwgZm9vZCBwcm9kdWN0aW9uLCB3aXRoICBpcnJpZ2F0ZWQgYWdyaWN1bHR1cmUgYWNjb3VudGluZyBmb3IgYXBwcm94aW1hdGVseSA0MCUgb2YgdGhlICBmb29kIHN1cHBseSAoV3JhY2hpZW4gZGUsIFNjaHVsdHogYW5kIEdvbGksIDIwMjE7IFNjaG1pdHQsIFJvc2EgIGFuZCBEYWlseSwgMjAyMikuIEhvd2V2ZXIsIHRyYWRpdGlvbmFsIGlycmlnYXRpb24gbWV0aG9kcywgc3VjaCBhcyAgZmxvb2QgYW5kIGZ1cnJvdyBpcnJpZ2F0aW9uLCBhcmUgb2Z0ZW4gaW5lZmZpY2llbnQsIGxlYWRpbmcgdG8gIHN1YnN0YW50aWFsIHdhdGVyIGxvc3NlcyB0aHJvdWdoIGV2YXBvcmF0aW9uLCBydW5vZmYsIGFuZCBkZWVwICBwZXJjb2xhdGlvbiAoQW1ib21zYSwgMjAyMCkuIE1vcmVvdmVyLCB0aGUgZXhjZXNzaXZlIGFwcGxpY2EtdGlvbiBvZiB3YXRlciBjYW4gcmVzdWx0IGluIHNvaWwgc2FsaW5pc2F0aW9uLCBudXRyaWVudCBsZWFjaGluZywgYW5kICBncm91bmR3YXRlciBkZXBsZXRpb24sIGZ1cnRoZXIgY29tcHJvbWlzaW5nIHRoZSBsb25nLXRlcm0gIHN1c3RhaW5hYmlsaXR5IG9mIGFncmljdWx0dXJhbCBzeXN0ZW1zIChZaW4gZXQgYWwuLCAyMDIyKS4gVG8gYWRkcmVzcyAgdGhlc2UgY2hhbGxlbmdlcywgcmVzZWFyY2hlcnMgYW5kIHByYWN0aXRpb25lcnMgaGF2ZSBiZWVuIGV4cGxvci1pbmcgaW5ub3ZhdGl2ZSBpcnJpZ2F0aW9uIHRlY2huaXF1ZXMgYW5kIHdhdGVyIG1hbmFnZW1lbnQgIHN0cmF0ZWdpZXMgdGhhdCBjYW4gb3B0aW1pc2Ugd2F0ZXIgdXNlIGVmZmljaWVuY3kgKFdVRSkgd2hpbGUgIG1haW50YWluaW5nIG9yIGVuaGFuY2luZyBjcm9wIHByb2R1Y3Rpdml0eS4gIFByZWNpc2lvbiBpcnJpZ2F0aW9uIChQSSkgb2ZmZXJzIGEgcHJvbWlzaW5nIHNvbHV0aW9uLCAgaW52b2x2aW5nIHRoZSB0YXJnZXRlZCBhcHBsaWNhdGlvbiBvZiB3YXRlciBzcGVjaWZpY2FsbHkgdG8gdGhlICBuZWVkcyBvZiBpbmRpdmlkdWFsIHBsYW50cyBvciBjcm9wIHpvbmVzIChaaW5rZXJuYWdlbCBldCBhbC4sIDIwMjApLiAgIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyMi44ODY3MzU5MTYxMzc2OTUsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzQ2MjIxOTIzODI4MTI1LCJoZWlnaHQiOjEuNDUwNTQxMDE5NDM5Njk3M319LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI0LjM1MTEyMzgwOTgxNDQ1MywibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTI3MTYwNjQ0NSwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI1LjcyNDk4NzAzMDAyOTI5NywibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTY1MzA3NjE3MiwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI3LjA5ODg1MDI1MDI0NDE0LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjE2LjQxODE2MTM5MjIxMTkxNCwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI4LjQ3MjcwOTY1NTc2MTcyLCJsZWZ0IjoxMi4zODA4ODAzNTU4MzQ5NjEsIndpZHRoIjozNy4wMTIxODQxNDMwNjY0MDYsImhlaWdodCI6MS4zNjAwMTQ3OTYyNTcwMTl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyOS44NDY1NzI4NzU5NzY1NjIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQwNjQ4NjUxMTIzMDUsImhlaWdodCI6MS4zNjAwMTY3MDM2MDU2NTE5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzEuMjE0MDM2OTQxNTI4MzIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQ0ODU2MjYyMjA3MDMsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzIuNTg3OTAyMDY5MDkxOCwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDYwNTc4OTE4NDU3LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMzLjk2MTc2MTQ3NDYwOTM3NSwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDA0ODQ2MTkxNDA2MjUsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzUuMzM1NjI0Njk0ODI0MjIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzM5NzQ0NTY3ODcxMDk0LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjM2LjcwOTQ4NzkxNTAzOTA2LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjAwMTU5MDcyODc1OTc2NiwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC4wODMzNDczMjA1NTY2NCwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTY1MzA3NjE3MiwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOS40NTcyMTA1NDA3NzE0ODQsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQzMDQ4MDk1NzAzMTI1LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQwLjc0MDU0NzE4MDE3NTc4LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjM0NTQ1NTE2OTY3NzczNCwiaGVpZ2h0IjoxLjQ1MDU0MjgwNzU3OTA0MDV9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Mi4yMDQ5MzMxNjY1MDM5MDYsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMDAzMjUwMTIyMDcwMzEsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDMuNTc4Nzk2Mzg2NzE4NzUsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzM3OTIxMTQyNTc4MTI1LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQ0Ljg2MjEzMzAyNjEyMzA1LCJsZWZ0Ijo5LjA0NzYwMTY5OTgyOTEwMiwid2lkdGgiOjQwLjM0NTMxNDAyNTg3ODkwNiwiaGVpZ2h0IjoxLjQ1MDU0MjgwNzU3OTA0MDV9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ni4zMjY1MTkwMTI0NTExNywibGVmdCI6OS4wNDc2MDE2OTk4MjkxMDIsIndpZHRoIjoyNy45Njc0NDcyODA4ODM3OSwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ny43MDAzODIyMzI2NjYwMTYsImxlZnQiOjEyLjM4MDg3ODQ0ODQ4NjMyOCwid2lkdGgiOjM3LjAzNDc4NjIyNDM2NTIzNCwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0OS4wNzQyNDU0NTI4ODA4NiwibGVmdCI6OS4wNDc2MDE2OTk4MjkxMDIsIndpZHRoIjo0MC4zNDA2NDQ4MzY0MjU3OCwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1MC4zNTc1ODIwOTIyODUxNTYsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzQ0ODUyNDQ3NTA5NzY2LCJoZWlnaHQiOjEuNDUwNTQyODA3NTc5MDQwNX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEzMSJ9fV0%3D;

- Data-driven insights: Enables predictive management.

### Remote Sensing and Satellite Imagery for Water Management

Remote sensing and satellite imagery enable large-scale, non-invasive monitoring of crop water status, soil moisture, and evapotranspiration rates, providing critical data for precision irrigation decisions that can boost water use efficiency by 20-30% through targeted applications (Jumintono et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikp1bWludG9ubyIsImZhbWlseSI6Ikp1bWludG9ubyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJOdXJuYXdhdHkiLCJmYW1pbHkiOiJOdXJuYXdhdHkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTnVncm9obyBUcmlzbnUiLCJmYW1pbHkiOiJCcmF0YSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJEaW5pIiwiZmFtaWx5IjoiUmFobWlhdGkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSGFkaSIsImZhbWlseSI6IlN1c2lsbyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBLiBBLiIsImZhbWlseSI6Ikh1c3NlbiJ9fV0sImlkcyI6eyJpc3NuIjpbIjE0MjktNzQyNiIsIjIwODMtNDUzNSJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMjQ0MjUvandsZC4yMDI0LjE1MTc5OCIsIm9wZW5hbGV4IjoiVzQ0MDU0NzcxMDAifSwidGl0bGUiOiJDb21wYXJhdGl2ZSBhbmFseXNpcyBvZiBwcmVjaXNpb24sIGRlZmljaXQsIGFuZCBkcmlwIGlycmlnYXRpb24gZm9yIHdhdGVyIGVmZmljaWVuY3kgaW4gSXJhcSIsImFic3RyYWN0IjoiU3VzdGFpbmFibGUgaXJyaWdhdGlvbiBwcmFjdGljZXMgYXJlIGNyaXRpY2FsIGZvciBhZGRyZXNzaW5nIHRoZSBjaGFsbGVuZ2VzIG9mIHdhdGVyIHNjYXJjaXR5IGFuZCBmb29kIGluc2VjdXJpdHkgaW4gdGhlIGZhY2Ugb2YgY2xpbWF0ZSBjaGFuZ2UgYW5kIGdyb3dpbmcgZ2xvYmFsIGZvb2QgZGVtYW5kLiBUaGlzIHN0dWR5IGludmVzdGlnYXRlZCB0aGUgcG90ZW50aWFsIG9mIHByZWNpc2lvbiBpcnJpZ2F0aW9uIChQSSksIGRlZmljaXQgaXJyaWdhdGlvbiAoREkpLCBhbmQgZHJpcCBpcnJpZ2F0aW9uIChEUkkpIHRlY2huaXF1ZXMgdG8gZW5oYW5jZSBjcm9wIHByb2R1Y3Rpdml0eSBhbmQgd2F0ZXIgdXNlIGVmZmljaWVuY3kgKFdVRSkgY29tcGFyZWQgdG8gY29udmVudGlvbmFsIGZsb29kIGlycmlnYXRpb24gKENGSSkgaW4gdGhyZWUgZGlzdGluY3QgYWdyb2Vjb2xvZ2ljYWwgem9uZXMgb2YgSXJhcS4gRmllbGQgZXhwZXJpbWVudHMgd2VyZSBjb25kdWN0ZWQgdXNpbmcgYSByYW5kb21pc2VkIGNvbXBsZXRlIGJsb2NrIGRlc2lnbiB3aXRoIHdoZWF0LCBtYWl6ZSwgYW5kIHJpY2UgY3JvcHMuIFRoZSByZXN1bHRzIGRlbW9uc3RyYXRlZCB0aGF0IFBJIHNpZ25pZmljYW50bHkgaW5jcmVhc2VkIGNyb3AgZ3Jvd3RoIHBhcmFtZXRlcnMsIGdyYWluIHlpZWxkLCBhbmQgV1VFIGFjcm9zcyBhbGwgem9uZXMsIHdpdGggeWllbGQgaW1wcm92ZW1lbnRzIG9mIDMz4oCTMzglIGFuZCBXVUUgaW5jcmVhc2VzIG9mIDQ24oCTNTElIGluIGNvbnRyYXN0IHdpdGggQ0ZJLiBUaGUgREkgYW5kIERSSSB0cmVhdG1lbnRzIGFsc28gb3V0cGVyZm9ybWVkIENGSSwgYWxiZWl0IHRvIGEgbGVzc2VyIGV4dGVudC4gUmVtb3RlIHNlbnNpbmctZGVyaXZlZCB2ZWdldGF0aW9uIGluZGljZXMgc3Ryb25nbHkgY29ycmVsYXRlZCB3aXRoIGNyb3AgZ3Jvd3RoIHBhcmFtZXRlcnMgYW5kIHlpZWxkLCB3aGlsZSBoeWRyb2xvZ2ljYWwgbW9kZWxsaW5nIHJldmVhbGVkIHJlZHVjZWQgZXZhcG90cmFuc3BpcmF0aW9uIGFuZCBzdXJmYWNlIHJ1bm9mZiB1bmRlciB0aGUgUEkgdHJlYXRtZW50LiBUaGUgc3VzdGFpbmFibGUgaXJyaWdhdGlvbiBwcmFjdGljZXMgcmVzdWx0ZWQgaW4gc3Vic3RhbnRpYWwgd2F0ZXIgc2F2aW5ncyBvZiAyMOKAkzMwJSBjb21wYXJlZCB0byBDRkkuIFRoZXNlIGZpbmRpbmdzIGhpZ2hsaWdodCB0aGUgaW1wb3J0YW5jZSBvZiBhZG9wdGluZyBlZmZpY2llbnQgaXJyaWdhdGlvbiB0ZWNobmlxdWVzLCBhbG9uZyB3aXRoIGEgaG9saXN0aWMgYXBwcm9hY2ggZW5jb21wYXNzaW5nIHRlY2hub2xvZ2ljYWwgaW5ub3ZhdGlvbnMsIGNhcGFjaXR5IGJ1aWxkaW5nLCBhbmQgc3Rha2Vob2xkZXIgZW5nYWdlbWVudCwgdG8gcHJvbW90ZSB3YXRlci0gZWZmaWNpZW50IGFncmljdWx0dXJlIGFuZCBlbnN1cmUgZm9vZCBzZWN1cml0eSBpbiB3YXRlci1zY2FyY2UgcmVnaW9ucy4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjQ0MjUvandsZC4yMDI0LjE1MTc5OCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjI0NDI1L2p3bGQuMjAyNC4xNTE3OTgiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMi0xNCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkpvdXJuYWwgb2YgV2F0ZXIgYW5kIExhbmQgRGV2ZWxvcG1lbnQiLCJpbXBhY3RGYWN0b3IiOiIxLjQ2In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTMwLCJsYXN0UGFnZSI6MTM2fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkRlIEdydXl0ZXIgT3BlbiJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MzIsImNpdGVkQnlDb3VudCI6MX19LCJxdW90ZSI6IkluIHRoaXMgY29udGV4dCwgdGhlIGFkb3B0aW9uIG9mICBzdXN0YWluYWJsZSBpcnJpZ2F0aW9uIHByYWN0aWNlcyBoYXMgZW1lcmdlZCBhcyBhIGNyaXRpY2FsIHN0cmF0ZWd5ICBmb3IgZW5oYW5jaW5nIGZvb2Qgc2VjdXJpdHkgd2hpbGUgcHJvbW90aW5nIHdhdGVyIGNvbnNlcnZhdGlvbiAgaW4gYWdyaWN1bHR1cmFsIGxhbmRzY2FwZXMuICBJcnJpZ2F0aW9uIHBsYXlzIGEgdml0YWwgcm9sZSBpbiBnbG9iYWwgZm9vZCBwcm9kdWN0aW9uLCB3aXRoICBpcnJpZ2F0ZWQgYWdyaWN1bHR1cmUgYWNjb3VudGluZyBmb3IgYXBwcm94aW1hdGVseSA0MCUgb2YgdGhlICBmb29kIHN1cHBseSAoV3JhY2hpZW4gZGUsIFNjaHVsdHogYW5kIEdvbGksIDIwMjE7IFNjaG1pdHQsIFJvc2EgIGFuZCBEYWlseSwgMjAyMikuIEhvd2V2ZXIsIHRyYWRpdGlvbmFsIGlycmlnYXRpb24gbWV0aG9kcywgc3VjaCBhcyAgZmxvb2QgYW5kIGZ1cnJvdyBpcnJpZ2F0aW9uLCBhcmUgb2Z0ZW4gaW5lZmZpY2llbnQsIGxlYWRpbmcgdG8gIHN1YnN0YW50aWFsIHdhdGVyIGxvc3NlcyB0aHJvdWdoIGV2YXBvcmF0aW9uLCBydW5vZmYsIGFuZCBkZWVwICBwZXJjb2xhdGlvbiAoQW1ib21zYSwgMjAyMCkuIE1vcmVvdmVyLCB0aGUgZXhjZXNzaXZlIGFwcGxpY2EtdGlvbiBvZiB3YXRlciBjYW4gcmVzdWx0IGluIHNvaWwgc2FsaW5pc2F0aW9uLCBudXRyaWVudCBsZWFjaGluZywgYW5kICBncm91bmR3YXRlciBkZXBsZXRpb24sIGZ1cnRoZXIgY29tcHJvbWlzaW5nIHRoZSBsb25nLXRlcm0gIHN1c3RhaW5hYmlsaXR5IG9mIGFncmljdWx0dXJhbCBzeXN0ZW1zIChZaW4gZXQgYWwuLCAyMDIyKS4gVG8gYWRkcmVzcyAgdGhlc2UgY2hhbGxlbmdlcywgcmVzZWFyY2hlcnMgYW5kIHByYWN0aXRpb25lcnMgaGF2ZSBiZWVuIGV4cGxvci1pbmcgaW5ub3ZhdGl2ZSBpcnJpZ2F0aW9uIHRlY2huaXF1ZXMgYW5kIHdhdGVyIG1hbmFnZW1lbnQgIHN0cmF0ZWdpZXMgdGhhdCBjYW4gb3B0aW1pc2Ugd2F0ZXIgdXNlIGVmZmljaWVuY3kgKFdVRSkgd2hpbGUgIG1haW50YWluaW5nIG9yIGVuaGFuY2luZyBjcm9wIHByb2R1Y3Rpdml0eS4gIFByZWNpc2lvbiBpcnJpZ2F0aW9uIChQSSkgb2ZmZXJzIGEgcHJvbWlzaW5nIHNvbHV0aW9uLCAgaW52b2x2aW5nIHRoZSB0YXJnZXRlZCBhcHBsaWNhdGlvbiBvZiB3YXRlciBzcGVjaWZpY2FsbHkgdG8gdGhlICBuZWVkcyBvZiBpbmRpdmlkdWFsIHBsYW50cyBvciBjcm9wIHpvbmVzIChaaW5rZXJuYWdlbCBldCBhbC4sIDIwMjApLiAgIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyMi44ODY3MzU5MTYxMzc2OTUsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzQ2MjIxOTIzODI4MTI1LCJoZWlnaHQiOjEuNDUwNTQxMDE5NDM5Njk3M319LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI0LjM1MTEyMzgwOTgxNDQ1MywibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTI3MTYwNjQ0NSwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI1LjcyNDk4NzAzMDAyOTI5NywibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTY1MzA3NjE3MiwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI3LjA5ODg1MDI1MDI0NDE0LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjE2LjQxODE2MTM5MjIxMTkxNCwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI4LjQ3MjcwOTY1NTc2MTcyLCJsZWZ0IjoxMi4zODA4ODAzNTU4MzQ5NjEsIndpZHRoIjozNy4wMTIxODQxNDMwNjY0MDYsImhlaWdodCI6MS4zNjAwMTQ3OTYyNTcwMTl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyOS44NDY1NzI4NzU5NzY1NjIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQwNjQ4NjUxMTIzMDUsImhlaWdodCI6MS4zNjAwMTY3MDM2MDU2NTE5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzEuMjE0MDM2OTQxNTI4MzIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQ0ODU2MjYyMjA3MDMsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzIuNTg3OTAyMDY5MDkxOCwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDYwNTc4OTE4NDU3LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMzLjk2MTc2MTQ3NDYwOTM3NSwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDA0ODQ2MTkxNDA2MjUsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzUuMzM1NjI0Njk0ODI0MjIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzM5NzQ0NTY3ODcxMDk0LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjM2LjcwOTQ4NzkxNTAzOTA2LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjAwMTU5MDcyODc1OTc2NiwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC4wODMzNDczMjA1NTY2NCwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTY1MzA3NjE3MiwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOS40NTcyMTA1NDA3NzE0ODQsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQzMDQ4MDk1NzAzMTI1LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQwLjc0MDU0NzE4MDE3NTc4LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjM0NTQ1NTE2OTY3NzczNCwiaGVpZ2h0IjoxLjQ1MDU0MjgwNzU3OTA0MDV9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Mi4yMDQ5MzMxNjY1MDM5MDYsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMDAzMjUwMTIyMDcwMzEsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDMuNTc4Nzk2Mzg2NzE4NzUsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzM3OTIxMTQyNTc4MTI1LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQ0Ljg2MjEzMzAyNjEyMzA1LCJsZWZ0Ijo5LjA0NzYwMTY5OTgyOTEwMiwid2lkdGgiOjQwLjM0NTMxNDAyNTg3ODkwNiwiaGVpZ2h0IjoxLjQ1MDU0MjgwNzU3OTA0MDV9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ni4zMjY1MTkwMTI0NTExNywibGVmdCI6OS4wNDc2MDE2OTk4MjkxMDIsIndpZHRoIjoyNy45Njc0NDcyODA4ODM3OSwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ny43MDAzODIyMzI2NjYwMTYsImxlZnQiOjEyLjM4MDg3ODQ0ODQ4NjMyOCwid2lkdGgiOjM3LjAzNDc4NjIyNDM2NTIzNCwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0OS4wNzQyNDU0NTI4ODA4NiwibGVmdCI6OS4wNDc2MDE2OTk4MjkxMDIsIndpZHRoIjo0MC4zNDA2NDQ4MzY0MjU3OCwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1MC4zNTc1ODIwOTIyODUxNTYsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzQ0ODUyNDQ3NTA5NzY2LCJoZWlnaHQiOjEuNDUwNTQyODA3NTc5MDQwNX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEzMSJ9fV0%3D. By integrating multispectral and thermal imagery from platforms like Landsat or Sentinel, farmers detect stress early, optimizing water allocation amid agriculture's dominant 70% share of global freshwater withdrawals (Daraz et al., 2025)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlVtYXIiLCJmYW1pbHkiOiJEYXJheiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiLFoHRlZmFuIiwiZmFtaWx5IjoiQm9qbmVjIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllvdW5hcyIsImZhbWlseSI6IktoYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMDc3LTA0NzIiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzOTAvYWdyaWN1bHR1cmUxNTA1MDU1NCIsIm9wZW5hbGV4IjoiVzQ0MDgyMTk4MTEiLCJleGEiOiJodHRwczovL3d3dy5tZHBpLmNvbS8yMDc3LTA0NzIvMTUvNS81NTQifSwidGl0bGUiOiJFbmVyZ3ktRWZmaWNpZW50IFNtYXJ0IElycmlnYXRpb24gVGVjaG5vbG9naWVzOiBBIFBhdGh3YXkgdG8gV2F0ZXIgYW5kIEVuZXJneSBTdXN0YWluYWJpbGl0eSBpbiBBZ3JpY3VsdHVyZSIsImFic3RyYWN0IjoiVGhlIGFncmljdWx0dXJhbCBzZWN0b3IgZmFjZXMgY2hhbGxlbmdlcyBzdWNoIGFzIHdhdGVyIHNjYXJjaXR5LCBlbmVyZ3kgaW5lZmZpY2llbmN5LCBhbmQgZGVjbGluaW5nIHByb2R1Y3Rpdml0eSwgcGFydGljdWxhcmx5IGluIGFyaWQgcmVnaW9ucy4gVHJhZGl0aW9uYWwgaXJyaWdhdGlvbiBtZXRob2RzIGNvbnRyaWJ1dGUgdG8gcmVzb3VyY2UgZGVwbGV0aW9uIGFuZCBlbnZpcm9ubWVudGFsIGltcGFjdHMuIFNvbGFyLXBvd2VyZWQgc21hcnQgaXJyaWdhdGlvbiBzeXN0ZW1zIGludGVncmF0ZSBwcmVjaXNpb24gaXJyaWdhdGlvbiB3aXRoIHJlbmV3YWJsZSBlbmVyZ3ksIGltcHJvdmluZyB3YXRlciB1c2UgYW5kIHByb2R1Y3Rpdml0eS4gSW4gUGFraXN0YW4sIHdoZXJlIGFncmljdWx0dXJlIGNvbnRyaWJ1dGVzIDE5JSBvZiBncm9zcyBkb21lc3RpYyBwcm9kdWN0IGFuZCBlbXBsb3lzIDQwJSBvZiB0aGUgd29ya2ZvcmNlLCB0aGVzZSBjaGFsbGVuZ2VzIGFyZSBzZXZlcmUsIGVzcGVjaWFsbHkgaW4gd2F0ZXItc2NhcmNlIGFyZWFzIGxpa2UgdGhlIENob2xpc3RhbiBEZXNlcnQuIFRoaXMgc3R1ZHkgZXhhbWluZXMgdGhlIGltcGFjdCBvZiBzb2xhci1wb3dlcmVkIHNtYXJ0IGlycmlnYXRpb24gb24gYWdyaWN1bHR1cmFsIHByb2R1Y3Rpdml0eSwgd2F0ZXIgY29uc2VydmF0aW9uLCBhbmQgZW5lcmd5IGVmZmljaWVuY3kgaW4gdGhlIENob2xpc3RhbiBEZXNlcnQuIFVzaW5nIGEgcXVhbnRpdGF0aXZlIGNyb3NzLXNlY3Rpb25hbCBkZXNpZ24sIGRhdGEgd2VyZSBjb2xsZWN0ZWQgZnJvbSAzODQgZmFybWVycyB2aWEgc3RydWN0dXJlZCBxdWVzdGlvbm5haXJlcy4gU3RhdGlzdGljYWwgYW5hbHlzZXMsIGluY2x1ZGluZyBtdWx0aXBsZSBsaW5lYXIgcmVncmVzc2lvbiwgcGFpcmVkIHNhbXBsZSB0LXRlc3RzLCBhbmQgU3RydWN0dXJhbCBFcXVhdGlvbiBNb2RlbGluZyAoU0VNKSwgd2VyZSBjb25kdWN0ZWQuIEZpbmRpbmdzIHNob3cgc2lnbmlmaWNhbnQgaW1wcm92ZW1lbnRzIGluIGNyb3AgeWllbGQgKGZyb20gMy4wIHRvIDQuOCB0b25zL2hlY3RhcmUpIGFuZCByZWR1Y3Rpb25zIGluIHdhdGVyIGFuZCBlbmVyZ3kgY29uc3VtcHRpb24uIFJlZ3Jlc3Npb24gYW5hbHlzaXMgaGlnaGxpZ2h0ZWQgc3Ryb25nIHBvc2l0aXZlIGVmZmVjdHMgb24geWllbGQgYW5kIGVmZmljaWVuY3ksIHdoaWxlIFNFTSBjb25maXJtZWQgcmVkdWNlZCBlbnZpcm9ubWVudGFsIGltcGFjdCBhbmQgb3BlcmF0aW9uYWwgY29zdHMuIFRoZSBzdHVkeSBjb25jbHVkZXMgdGhhdCBzb2xhci1wb3dlcmVkIGlycmlnYXRpb24gZW5oYW5jZXMgcHJvZHVjdGl2aXR5LCBjb25zZXJ2ZXMgcmVzb3VyY2VzLCBhbmQgcHJvbW90ZXMgc3VzdGFpbmFiaWxpdHkuIFBvbGljeW1ha2VycyBzaG91bGQgcHJvdmlkZSBmaW5hbmNpYWwgaW5jZW50aXZlcywgaW52ZXN0IGluIHJlbmV3YWJsZSBpbmZyYXN0cnVjdHVyZSwgYW5kIGltcGxlbWVudCB0cmFpbmluZyBwcm9ncmFtcyB0byBzdXBwb3J0IGFkb3B0aW9uLiBDb2xsYWJvcmF0aXZlIGVmZm9ydHMgYXJlIGVzc2VudGlhbCBmb3Igc3VzdGFpbmFibGUgYWdyaWN1bHR1cmUgaW4gYXJpZCByZWdpb25zLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJpc3N1ZWREYXRlIjoiMjAyNS0wMy0wNSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkFncmljdWx0dXJlIiwiaXNzdWUiOjUsInZvbHVtZSI6MTUsImltcGFjdEZhY3RvciI6IjUuMjYifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjo1NTQsImxhc3RQYWdlIjo1NTR9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiTXVsdGlkaXNjaXBsaW5hcnkgRGlnaXRhbCBQdWJsaXNoaW5nIEluc3RpdHV0ZSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NDgsImNpdGVkQnlDb3VudCI6Mn19LCJxdW90ZSI6IjEuIEludHJvZHVjdGlvbiBUaGUgYWdyaWN1bHR1cmFsIHNlY3RvciwgYSBjb3JuZXJzdG9uZSBvZiBnbG9iYWwgZm9vZCBzZWN1cml0eSBhbmQgZWNvbm9taWMgZGV2ZWxvcG1lbnQsIGZhY2VzIG1vdW50aW5nIGNoYWxsZW5nZXMgaW4gdGhlIDIxc3QgY2VudHVyeS4gQ2xpbWF0ZSBjaGFuZ2UsIHJhcGlkIHBvcHVsYXRpb24gZ3Jvd3RoLCBhbmQgZHdpbmRsaW5nIG5hdHVyYWwgcmVzb3VyY2VzIGhhdmUgaW50ZW5zaWZpZWQgdGhlIG5lZWQgZm9yIHN1c3RhaW5hYmxlIGFncmljdWx0dXJhbCBwcmFjdGljZXMuIEFtb25nIHRoZXNlIGNoYWxsZW5nZXMsIHdhdGVyIHNjYXJjaXR5IGFuZCBlbmVyZ3kgaW5lZmZpY2llbmN5IHN0YW5kIG91dCBhcyBjcml0aWNhbCBiYXJyaWVycyB0byBwcm9kdWN0aXZpdHkgYW5kIHN1c3RhaW5hYmlsaXR5LiBHbG9iYWxseSwgYWdyaWN1bHR1cmUgYWNjb3VudHMgZm9yIGFwcHJveGltYXRlbHkgNzAlIG9mIGZyZXNod2F0ZXIgd2l0aGRyYXdhbHMgYW5kIGlzIG9uZSBvZiB0aGUgbGFyZ2VzdCBjb25zdW1lcnMgb2YgZW5lcmd5IGluIG1hbnkgcmVnaW9ucyBbIDEsIDJdLiBUcmFkaXRpb25hbCBpcnJpZ2F0aW9uIHByYWN0aWNlcywgb2Z0ZW4gaW5lZmZpY2llbnQgYW5kIHJlc291cmNlLWludGVuc2l2ZSwgZXhhY2VyYmF0ZSB0aGVzZSBpc3N1ZXMsIGxlYWRpbmcgdG8gd2F0ZXIgd2FzdGFnZSwgZW5lcmd5IG92ZXJ1c2UsIGFuZCByZWR1Y2VkIGNyb3AgeWllbGRzLiJ9XQ%3D%3D.

### Table 6: Global Agricultural Pressures (21st Century)

| Pressure Factor | Impact on Agriculture | Resulting Requirement |
| --- | --- | --- |
| Climate Change | Unpredictable weather | Adaptive technologies |
| Population Growth | Escalating food demand | Increased productivity |
| Water Scarcity | Depleted groundwater | Efficient use efficiency |
| Resource Limits | Finite natural assets | Sustainable management |
| Energy Costs | High operational burden | Energy-efficient systems |
| Land Degradation | Reduced soil health | Conservation practices |
| Biodiversity Loss | Ecosystem imbalance | Sustainable alternatives |
| Economic Stability | High input costs | Precision resource use |

Principles of Operation

Satellites capture reflectance data to compute indices such as NDVI and CWSI, processed via cloud-based analytics for real-time ET maps. Ground validation with drones enhances resolution for field-scale insights (Daraz et al., 2025)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlVtYXIiLCJmYW1pbHkiOiJEYXJheiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiLFoHRlZmFuIiwiZmFtaWx5IjoiQm9qbmVjIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllvdW5hcyIsImZhbWlseSI6IktoYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMDc3LTA0NzIiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzOTAvYWdyaWN1bHR1cmUxNTA1MDU1NCIsIm9wZW5hbGV4IjoiVzQ0MDgyMTk4MTEiLCJleGEiOiJodHRwczovL3d3dy5tZHBpLmNvbS8yMDc3LTA0NzIvMTUvNS81NTQifSwidGl0bGUiOiJFbmVyZ3ktRWZmaWNpZW50IFNtYXJ0IElycmlnYXRpb24gVGVjaG5vbG9naWVzOiBBIFBhdGh3YXkgdG8gV2F0ZXIgYW5kIEVuZXJneSBTdXN0YWluYWJpbGl0eSBpbiBBZ3JpY3VsdHVyZSIsImFic3RyYWN0IjoiVGhlIGFncmljdWx0dXJhbCBzZWN0b3IgZmFjZXMgY2hhbGxlbmdlcyBzdWNoIGFzIHdhdGVyIHNjYXJjaXR5LCBlbmVyZ3kgaW5lZmZpY2llbmN5LCBhbmQgZGVjbGluaW5nIHByb2R1Y3Rpdml0eSwgcGFydGljdWxhcmx5IGluIGFyaWQgcmVnaW9ucy4gVHJhZGl0aW9uYWwgaXJyaWdhdGlvbiBtZXRob2RzIGNvbnRyaWJ1dGUgdG8gcmVzb3VyY2UgZGVwbGV0aW9uIGFuZCBlbnZpcm9ubWVudGFsIGltcGFjdHMuIFNvbGFyLXBvd2VyZWQgc21hcnQgaXJyaWdhdGlvbiBzeXN0ZW1zIGludGVncmF0ZSBwcmVjaXNpb24gaXJyaWdhdGlvbiB3aXRoIHJlbmV3YWJsZSBlbmVyZ3ksIGltcHJvdmluZyB3YXRlciB1c2UgYW5kIHByb2R1Y3Rpdml0eS4gSW4gUGFraXN0YW4sIHdoZXJlIGFncmljdWx0dXJlIGNvbnRyaWJ1dGVzIDE5JSBvZiBncm9zcyBkb21lc3RpYyBwcm9kdWN0IGFuZCBlbXBsb3lzIDQwJSBvZiB0aGUgd29ya2ZvcmNlLCB0aGVzZSBjaGFsbGVuZ2VzIGFyZSBzZXZlcmUsIGVzcGVjaWFsbHkgaW4gd2F0ZXItc2NhcmNlIGFyZWFzIGxpa2UgdGhlIENob2xpc3RhbiBEZXNlcnQuIFRoaXMgc3R1ZHkgZXhhbWluZXMgdGhlIGltcGFjdCBvZiBzb2xhci1wb3dlcmVkIHNtYXJ0IGlycmlnYXRpb24gb24gYWdyaWN1bHR1cmFsIHByb2R1Y3Rpdml0eSwgd2F0ZXIgY29uc2VydmF0aW9uLCBhbmQgZW5lcmd5IGVmZmljaWVuY3kgaW4gdGhlIENob2xpc3RhbiBEZXNlcnQuIFVzaW5nIGEgcXVhbnRpdGF0aXZlIGNyb3NzLXNlY3Rpb25hbCBkZXNpZ24sIGRhdGEgd2VyZSBjb2xsZWN0ZWQgZnJvbSAzODQgZmFybWVycyB2aWEgc3RydWN0dXJlZCBxdWVzdGlvbm5haXJlcy4gU3RhdGlzdGljYWwgYW5hbHlzZXMsIGluY2x1ZGluZyBtdWx0aXBsZSBsaW5lYXIgcmVncmVzc2lvbiwgcGFpcmVkIHNhbXBsZSB0LXRlc3RzLCBhbmQgU3RydWN0dXJhbCBFcXVhdGlvbiBNb2RlbGluZyAoU0VNKSwgd2VyZSBjb25kdWN0ZWQuIEZpbmRpbmdzIHNob3cgc2lnbmlmaWNhbnQgaW1wcm92ZW1lbnRzIGluIGNyb3AgeWllbGQgKGZyb20gMy4wIHRvIDQuOCB0b25zL2hlY3RhcmUpIGFuZCByZWR1Y3Rpb25zIGluIHdhdGVyIGFuZCBlbmVyZ3kgY29uc3VtcHRpb24uIFJlZ3Jlc3Npb24gYW5hbHlzaXMgaGlnaGxpZ2h0ZWQgc3Ryb25nIHBvc2l0aXZlIGVmZmVjdHMgb24geWllbGQgYW5kIGVmZmljaWVuY3ksIHdoaWxlIFNFTSBjb25maXJtZWQgcmVkdWNlZCBlbnZpcm9ubWVudGFsIGltcGFjdCBhbmQgb3BlcmF0aW9uYWwgY29zdHMuIFRoZSBzdHVkeSBjb25jbHVkZXMgdGhhdCBzb2xhci1wb3dlcmVkIGlycmlnYXRpb24gZW5oYW5jZXMgcHJvZHVjdGl2aXR5LCBjb25zZXJ2ZXMgcmVzb3VyY2VzLCBhbmQgcHJvbW90ZXMgc3VzdGFpbmFiaWxpdHkuIFBvbGljeW1ha2VycyBzaG91bGQgcHJvdmlkZSBmaW5hbmNpYWwgaW5jZW50aXZlcywgaW52ZXN0IGluIHJlbmV3YWJsZSBpbmZyYXN0cnVjdHVyZSwgYW5kIGltcGxlbWVudCB0cmFpbmluZyBwcm9ncmFtcyB0byBzdXBwb3J0IGFkb3B0aW9uLiBDb2xsYWJvcmF0aXZlIGVmZm9ydHMgYXJlIGVzc2VudGlhbCBmb3Igc3VzdGFpbmFibGUgYWdyaWN1bHR1cmUgaW4gYXJpZCByZWdpb25zLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJpc3N1ZWREYXRlIjoiMjAyNS0wMy0wNSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkFncmljdWx0dXJlIiwiaXNzdWUiOjUsInZvbHVtZSI6MTUsImltcGFjdEZhY3RvciI6IjUuMjYifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjo1NTQsImxhc3RQYWdlIjo1NTR9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiTXVsdGlkaXNjaXBsaW5hcnkgRGlnaXRhbCBQdWJsaXNoaW5nIEluc3RpdHV0ZSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NDgsImNpdGVkQnlDb3VudCI6Mn19LCJxdW90ZSI6IjEuIEludHJvZHVjdGlvbiBUaGUgYWdyaWN1bHR1cmFsIHNlY3RvciwgYSBjb3JuZXJzdG9uZSBvZiBnbG9iYWwgZm9vZCBzZWN1cml0eSBhbmQgZWNvbm9taWMgZGV2ZWxvcG1lbnQsIGZhY2VzIG1vdW50aW5nIGNoYWxsZW5nZXMgaW4gdGhlIDIxc3QgY2VudHVyeS4gQ2xpbWF0ZSBjaGFuZ2UsIHJhcGlkIHBvcHVsYXRpb24gZ3Jvd3RoLCBhbmQgZHdpbmRsaW5nIG5hdHVyYWwgcmVzb3VyY2VzIGhhdmUgaW50ZW5zaWZpZWQgdGhlIG5lZWQgZm9yIHN1c3RhaW5hYmxlIGFncmljdWx0dXJhbCBwcmFjdGljZXMuIEFtb25nIHRoZXNlIGNoYWxsZW5nZXMsIHdhdGVyIHNjYXJjaXR5IGFuZCBlbmVyZ3kgaW5lZmZpY2llbmN5IHN0YW5kIG91dCBhcyBjcml0aWNhbCBiYXJyaWVycyB0byBwcm9kdWN0aXZpdHkgYW5kIHN1c3RhaW5hYmlsaXR5LiBHbG9iYWxseSwgYWdyaWN1bHR1cmUgYWNjb3VudHMgZm9yIGFwcHJveGltYXRlbHkgNzAlIG9mIGZyZXNod2F0ZXIgd2l0aGRyYXdhbHMgYW5kIGlzIG9uZSBvZiB0aGUgbGFyZ2VzdCBjb25zdW1lcnMgb2YgZW5lcmd5IGluIG1hbnkgcmVnaW9ucyBbIDEsIDJdLiBUcmFkaXRpb25hbCBpcnJpZ2F0aW9uIHByYWN0aWNlcywgb2Z0ZW4gaW5lZmZpY2llbnQgYW5kIHJlc291cmNlLWludGVuc2l2ZSwgZXhhY2VyYmF0ZSB0aGVzZSBpc3N1ZXMsIGxlYWRpbmcgdG8gd2F0ZXIgd2FzdGFnZSwgZW5lcmd5IG92ZXJ1c2UsIGFuZCByZWR1Y2VkIGNyb3AgeWllbGRzLiJ9XQ%3D%3D.

Advantages and Efficiency Gains

- Data scalability: Covers vast areas cost-effectively, ideal for heterogeneous fields (Kushwaha et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjMwLCJzdWJ0eXBlIjp7fSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTWFuaXNoIiwiZmFtaWx5IjoiS3VzaHdhaGEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2hhbmthciIsImZhbWlseSI6IlNpbmdoIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlZpamF5IEthbnQiLCJmYW1pbHkiOiJTaW5naCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTaGFzaGFuayIsImZhbWlseSI6IkR3aXZlZGkifX1dLCJpZHMiOnsiaXNzbiI6WyIyNDU2LTE4NzgiXSwiaXNibiI6W10sImRvaSI6IjEwLjIyMTYxL2lqZWFiLjkyLjI3Iiwib3BlbmFsZXgiOiJXNDM5NjU2MzU4NCJ9LCJ0aXRsZSI6IlByZWNpc2lvbiBGYXJtaW5nOiBBIFJldmlldyBvZiBNZXRob2RzLCBUZWNobm9sb2dpZXMsIGFuZCBGdXR1cmUgUHJvc3BlY3RzIiwiYWJzdHJhY3QiOiJQcmVjaXNpb24gZmFybWluZywgb2Z0ZW4gcmVmZXJyZWQgdG8gYXMgcHJlY2lzaW9uIGFncmljdWx0dXJlLCBpcyBhIGZhcm1pbmcgbWFuYWdlbWVudCBjb25jZXB0IHRoYXQgc2Vla3MgdG8gbWF4aW1pemUgYWdyaWN1bHR1cmFsIG9wZXJhdGlvbnMgdmlhIHRoZSB1c2Ugb2YgdGVjaG5vbG9neSBpbiBvcmRlciB0byBhY2hpZXZlIGdyZWF0ZXIgbGV2ZWxzIG9mIGVmZmljaWVuY3ksIHByb2R1Y3Rpb24sIGFuZCBjb25zZXJ2YXRpb24gb2YgcmVzb3VyY2VzLiBUaGUgcHVycG9zZSBvZiB0aGlzIHN1cnZleSBzdHVkeSBpcyB0byBvZmZlciBhbiBvdmVydmlldyBvZiBwcmVjaXNpb24gZmFybWluZywgaW5jbHVkaW5nIHRvcGljcyBzdWNoIGFzIGl0cyBmdW5kYW1lbnRhbHMsIG1ldGhvZG9sb2dpZXMsIHRlY2hub2xvZ3ksIGFuZCBhcHBsaWNhdGlvbnMuIEl0IGludmVzdGlnYXRlcyB0aGUgcm9sZSB0aGF0IGEgdmFyaWV0eSBvZiB0ZWNobm9sb2dpZXMsIGluY2x1ZGluZyBnbG9iYWwgbmF2aWdhdGlvbiBzYXRlbGxpdGUgc3lzdGVtcyAoR05TUyksIGdlb2dyYXBoaWMgaW5mb3JtYXRpb24gc3lzdGVtcyAoR0lTKSwgcmVtb3RlIHNlbnNpbmcsIGFuZCBzZW5zb3IgdGVjaG5vbG9neSwgcGxheSBpbiBtYWtpbmcgaXQgcG9zc2libGUgdG8gaW1wbGVtZW50IHByZWNpc2lvbiBhZ3JpY3VsdHVyYWwgbWV0aG9kcy4gSW4gYWRkaXRpb24sIGl0IGFkZHJlc3NlcyB0aGUgYWR2YW50YWdlcywgZGlzYWR2YW50YWdlcywgYW5kIHBvdGVudGlhbCBmdXR1cmUgYXBwbGljYXRpb25zIG9mIHByZWNpc2lvbiBmYXJtaW5nIGluIHRoZSBjb250ZXh0IG9mIHNvbHZpbmcgaXNzdWVzIHJlbGF0ZWQgdG8gdGhlIHN1c3RhaW5hYmlsaXR5IG9mIGFncmljdWx0dXJlIGFuZCB0aGUgc2VjdXJpdHkgb2YgZm9vZCBzdXBwbGllcyBvbiBhIHdvcmxkd2lkZSBzY2FsZS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjIxNjEvaWplYWIuOTIuMjciLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vaWplYWIuY29tL3VwbG9hZF9kb2N1bWVudC9pc3N1ZV9maWxlcy8yN0lKRUFCLTEwNDIwMjQ4NC1QcmVjaXNpb25GYXJtaW5nLnBkZiIsImlzc3VlZERhdGUiOiIyMDI0LTAxLTAxIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiSW50ZXJuYXRpb25hbCBKb3VybmFsIG9mIEVudmlyb25tZW50IEFncmljdWx0dXJlIGFuZCBCaW90ZWNobm9sb2d5IiwiaXNzdWUiOjIsInZvbHVtZSI6OSwiaW1wYWN0RmFjdG9yIjoiMC42NSJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjI0MiwibGFzdFBhZ2UiOjI1M319LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJJbmZvZ2FpbiBQdWJsaWNhdGlvbiJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NCwiY2l0ZWRCeUNvdW50Ijo4fX0sInF1b3RlIjoiSUlJLiBCRU5FRklUUyBPRiBQUkVDSVNJT04gSVJSSUdBVElPTiAxLiBJbXByb3ZlZCBXYXRlciBVc2UgRWZmaWNpZW5jeTogUHJlY2lzaW9uIGlycmlnYXRpb24gb3B0aW1pemVzIHdhdGVyIHVzZSBlZmZpY2llbmN5IGJ5IGRlbGl2ZXJpbmcgd2F0ZXIgZGlyZWN0bHkgdG8gdGhlIHJvb3Qgem9uZSBvZiBjcm9wcywgbWluaW1pemluZyBsb3NzZXMgIGR1ZSB0byBldmFwb3JhdGlvbiwgcnVub2ZmLCBhbmQgZGVlcCBwZXJjb2xhdGlvbi4gQnkgIG1hdGNoaW5nIHdhdGVyIGFwcGxpY2F0aW9uIHJhdGVzIHRvIGNyb3Agd2F0ZXIgIHJlcXVpcmVtZW50cywgcHJlY2lzaW9uIGlycmlnYXRpb24gc3lzdGVtcyBlbnN1cmUgdGhhdCAgd2F0ZXIgaXMgdXNlZCBtb3JlIGVmZmljaWVudGx5LCBtYXhpbWl6aW5nIGNyb3AgeWllbGRzICBwZXIgdW5pdCBvZiB3YXRlciBhcHBsaWVkLiAyLiBFbmhhbmNlZCBDcm9wIFBlcmZvcm1hbmNlOiBCeSBwcm92aWRpbmcgY3JvcHMgd2l0aCBvcHRpbWFsIG1vaXN0dXJlIGxldmVscywgcHJlY2lzaW9uIGlycmlnYXRpb24gc3lzdGVtcyBwcm9tb3RlIGhlYWx0aHkgcm9vdCBkZXZlbG9wbWVudCwgYmFsYW5jZWQgZ3Jvd3RoLCAgYW5kIGltcHJvdmVkIGNyb3Agdmlnb3IuIENvbnNpc3RlbnQgc29pbCBtb2lzdHVyZSAgbGV2ZWxzIGhlbHAgcmVkdWNlIHdhdGVyIHN0cmVzcywgaW5jcmVhc2UgbnV0cmllbnQgIHVwdGFrZSwgYW5kIGVuaGFuY2UgY3JvcCByZXNpbGllbmNlIHRvIGVudmlyb25tZW50YWwgIHN0cmVzc2VzIHN1Y2ggYXMgZHJvdWdodCBhbmQgaGVhdC4gMy4gQ29zdCBTYXZpbmdzOiBQcmVjaXNpb24gaXJyaWdhdGlvbiBzeXN0ZW1zIGhlbHAgZmFybWVycyByZWR1Y2Ugd2F0ZXIsIGVuZXJneSwgYW5kIGxhYm9yIGNvc3RzIGFzc29jaWF0ZWQgd2l0aCBpcnJpZ2F0aW9uLiBCeSBvcHRpbWl6aW5nIHdhdGVyIHVzZSBlZmZpY2llbmN5IGFuZCAgbWluaW1pemluZyBpbnB1dCB3YXN0YWdlLCBwcmVjaXNpb24gaXJyaWdhdGlvbiBzeXN0ZW1zICBjYW4gbGVhZCB0byBzaWduaWZpY2FudCBjb3N0IHNhdmluZ3Mgb3ZlciB0aW1lLCAgaW1wcm92aW5nIHRoZSBwcm9maXRhYmlsaXR5IG9mIGFncmljdWx0dXJhbCBvcGVyYXRpb25zLiA0LiBFbnZpcm9ubWVudGFsIFN1c3RhaW5hYmlsaXR5OiAgUHJlY2lzaW9uICBpcnJpZ2F0aW9uIGNvbnRyaWJ1dGVzIHRvICBlbnZpcm9ubWVudGFsICBzdXN0YWluYWJpbGl0eSAgYnkgY29uc2VydmluZyB3YXRlciAgcmVzb3VyY2VzLCAgbWluaW1pemluZyAgd2F0ZXIgcG9sbHV0aW9uLCBhbmQgcmVkdWNpbmcgZ3JlZW5ob3VzZSBnYXMgZW1pc3Npb25zLiBCeSBhZG9wdGluZyBwcmVjaXNpb24gaXJyaWdhdGlvbiBwcmFjdGljZXMsIGZhcm1lcnMgY2FuICBtaW5pbWl6ZSB0aGVpciBlbnZpcm9ubWVudGFsIGZvb3RwcmludCBhbmQgY29udHJpYnV0ZSB0byB0aGUgIGxvbmctdGVybSAgc3VzdGFpbmFiaWxpdHkgIG9mICBhZ3JpY3VsdHVyYWwgcHJvZHVjdGlvbiBzeXN0ZW1zLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjQ0Ljg2NTY0MjU0NzYwNzQyLCJsZWZ0Ijo1My40MDgyNTI3MTYwNjQ0NSwid2lkdGgiOjM4LjI2MTc2ODM0MTA2NDQ1LCJoZWlnaHQiOjEuNjY5MjI3MzYxNjc5MDc3MX19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjQ2Ljg5NzkyMjUxNTg2OTE0LCJsZWZ0Ijo1Mi40MjA1NDM2NzA2NTQzLCJ3aWR0aCI6NDAuMjQwNzM3OTE1MDM5MDYsImhlaWdodCI6MS42MjU0NTQ3ODM0Mzk2MzYyfX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NDguNDkxNzkwNzcxNDg0Mzc1LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3Ljk4MDI3ODAxNTEzNjcyLCJoZWlnaHQiOjExLjAxMzcxMzgzNjY2OTkyMn19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjU5LjkzOTU0MDg2MzAzNzExLCJsZWZ0Ijo1Mi40MjA1NDM2NzA2NTQzLCJ3aWR0aCI6NDAuMjMwMjI0NjA5Mzc1LCJoZWlnaHQiOjEuNjI1NDU0NzgzNDM5NjM2Mn19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjYxLjUzMzQxNjc0ODA0Njg3NSwibGVmdCI6NTQuNjk4MzEwODUyMDUwNzgsIndpZHRoIjozNy45NjMxMjMzMjE1MzMyLCJoZWlnaHQiOjkuNDM1MTcwMTczNjQ1MDJ9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo3MS40MTY4Nzc3NDY1ODIwMywibGVmdCI6NTIuNDIwNTQzNjcwNjU0Mywid2lkdGgiOjQwLjIxMzMxMDI0MTY5OTIyLCJoZWlnaHQiOjEuNjI1NDU4MzU5NzE4MzIyOH19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjczLjAxMDc1NzQ0NjI4OTA2LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3Ljk0NTc4NTUyMjQ2MDk0LCJoZWlnaHQiOjcuODYzNzY0Mjg2MDQxMjZ9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo4MS4zMDg1NTU2MDMwMjczNCwibGVmdCI6NTIuNDIwNTQzNjcwNjU0Mywid2lkdGgiOjQwLjI0MDM3NTUxODc5ODgzLCJoZWlnaHQiOjEuNjI1NDU4MzU5NzE4MzIyOH19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjgyLjkxOTA1OTc1MzQxNzk3LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3LjkzNjQxNjYyNTk3NjU2LCJoZWlnaHQiOjEuNTc4MTMzNTgzMDY4ODQ3N319LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjg0LjQ4NjkwMDMyOTU4OTg0LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3LjkzNDc1NzIzMjY2NjAxNiwiaGVpZ2h0IjoxLjU3ODEzMzU4MzA2ODg0Nzd9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo4Ni4wNTQ3NDg1MzUxNTYyNSwibGVmdCI6NTQuNjk4MzEwODUyMDUwNzgsIndpZHRoIjozNy45Nzg0MzE3MDE2NjAxNTYsImhlaWdodCI6NC43MTM4MTkwMjY5NDcwMjE1fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6OC40MjAyOTI4NTQzMDkwODIsImxlZnQiOjEwLjAyMjE3MjkyNzg1NjQ0NSwid2lkdGgiOjM3Ljk0NjgzMDc0OTUxMTcyLCJoZWlnaHQiOjEuNTc4MTM1MjUxOTk4OTAxNH19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjkuOTg4MTMyNDc2ODA2NjQsImxlZnQiOjEwLjAyMjE3MjkyNzg1NjQ0NSwid2lkdGgiOjEzLjg2MjQ2MTA5MDA4Nzg5LCJoZWlnaHQiOjEuNTc4MTM1MjUxOTk4OTAxNH19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjI0OSJ9fV0%3D;

- Predictive accuracy: Reduces over-irrigation by forecasting deficits, saving 25-40% water (Jumintono et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikp1bWludG9ubyIsImZhbWlseSI6Ikp1bWludG9ubyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJOdXJuYXdhdHkiLCJmYW1pbHkiOiJOdXJuYXdhdHkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTnVncm9obyBUcmlzbnUiLCJmYW1pbHkiOiJCcmF0YSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJEaW5pIiwiZmFtaWx5IjoiUmFobWlhdGkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSGFkaSIsImZhbWlseSI6IlN1c2lsbyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBLiBBLiIsImZhbWlseSI6Ikh1c3NlbiJ9fV0sImlkcyI6eyJpc3NuIjpbIjE0MjktNzQyNiIsIjIwODMtNDUzNSJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMjQ0MjUvandsZC4yMDI0LjE1MTc5OCIsIm9wZW5hbGV4IjoiVzQ0MDU0NzcxMDAifSwidGl0bGUiOiJDb21wYXJhdGl2ZSBhbmFseXNpcyBvZiBwcmVjaXNpb24sIGRlZmljaXQsIGFuZCBkcmlwIGlycmlnYXRpb24gZm9yIHdhdGVyIGVmZmljaWVuY3kgaW4gSXJhcSIsImFic3RyYWN0IjoiU3VzdGFpbmFibGUgaXJyaWdhdGlvbiBwcmFjdGljZXMgYXJlIGNyaXRpY2FsIGZvciBhZGRyZXNzaW5nIHRoZSBjaGFsbGVuZ2VzIG9mIHdhdGVyIHNjYXJjaXR5IGFuZCBmb29kIGluc2VjdXJpdHkgaW4gdGhlIGZhY2Ugb2YgY2xpbWF0ZSBjaGFuZ2UgYW5kIGdyb3dpbmcgZ2xvYmFsIGZvb2QgZGVtYW5kLiBUaGlzIHN0dWR5IGludmVzdGlnYXRlZCB0aGUgcG90ZW50aWFsIG9mIHByZWNpc2lvbiBpcnJpZ2F0aW9uIChQSSksIGRlZmljaXQgaXJyaWdhdGlvbiAoREkpLCBhbmQgZHJpcCBpcnJpZ2F0aW9uIChEUkkpIHRlY2huaXF1ZXMgdG8gZW5oYW5jZSBjcm9wIHByb2R1Y3Rpdml0eSBhbmQgd2F0ZXIgdXNlIGVmZmljaWVuY3kgKFdVRSkgY29tcGFyZWQgdG8gY29udmVudGlvbmFsIGZsb29kIGlycmlnYXRpb24gKENGSSkgaW4gdGhyZWUgZGlzdGluY3QgYWdyb2Vjb2xvZ2ljYWwgem9uZXMgb2YgSXJhcS4gRmllbGQgZXhwZXJpbWVudHMgd2VyZSBjb25kdWN0ZWQgdXNpbmcgYSByYW5kb21pc2VkIGNvbXBsZXRlIGJsb2NrIGRlc2lnbiB3aXRoIHdoZWF0LCBtYWl6ZSwgYW5kIHJpY2UgY3JvcHMuIFRoZSByZXN1bHRzIGRlbW9uc3RyYXRlZCB0aGF0IFBJIHNpZ25pZmljYW50bHkgaW5jcmVhc2VkIGNyb3AgZ3Jvd3RoIHBhcmFtZXRlcnMsIGdyYWluIHlpZWxkLCBhbmQgV1VFIGFjcm9zcyBhbGwgem9uZXMsIHdpdGggeWllbGQgaW1wcm92ZW1lbnRzIG9mIDMz4oCTMzglIGFuZCBXVUUgaW5jcmVhc2VzIG9mIDQ24oCTNTElIGluIGNvbnRyYXN0IHdpdGggQ0ZJLiBUaGUgREkgYW5kIERSSSB0cmVhdG1lbnRzIGFsc28gb3V0cGVyZm9ybWVkIENGSSwgYWxiZWl0IHRvIGEgbGVzc2VyIGV4dGVudC4gUmVtb3RlIHNlbnNpbmctZGVyaXZlZCB2ZWdldGF0aW9uIGluZGljZXMgc3Ryb25nbHkgY29ycmVsYXRlZCB3aXRoIGNyb3AgZ3Jvd3RoIHBhcmFtZXRlcnMgYW5kIHlpZWxkLCB3aGlsZSBoeWRyb2xvZ2ljYWwgbW9kZWxsaW5nIHJldmVhbGVkIHJlZHVjZWQgZXZhcG90cmFuc3BpcmF0aW9uIGFuZCBzdXJmYWNlIHJ1bm9mZiB1bmRlciB0aGUgUEkgdHJlYXRtZW50LiBUaGUgc3VzdGFpbmFibGUgaXJyaWdhdGlvbiBwcmFjdGljZXMgcmVzdWx0ZWQgaW4gc3Vic3RhbnRpYWwgd2F0ZXIgc2F2aW5ncyBvZiAyMOKAkzMwJSBjb21wYXJlZCB0byBDRkkuIFRoZXNlIGZpbmRpbmdzIGhpZ2hsaWdodCB0aGUgaW1wb3J0YW5jZSBvZiBhZG9wdGluZyBlZmZpY2llbnQgaXJyaWdhdGlvbiB0ZWNobmlxdWVzLCBhbG9uZyB3aXRoIGEgaG9saXN0aWMgYXBwcm9hY2ggZW5jb21wYXNzaW5nIHRlY2hub2xvZ2ljYWwgaW5ub3ZhdGlvbnMsIGNhcGFjaXR5IGJ1aWxkaW5nLCBhbmQgc3Rha2Vob2xkZXIgZW5nYWdlbWVudCwgdG8gcHJvbW90ZSB3YXRlci0gZWZmaWNpZW50IGFncmljdWx0dXJlIGFuZCBlbnN1cmUgZm9vZCBzZWN1cml0eSBpbiB3YXRlci1zY2FyY2UgcmVnaW9ucy4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjQ0MjUvandsZC4yMDI0LjE1MTc5OCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjI0NDI1L2p3bGQuMjAyNC4xNTE3OTgiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMi0xNCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkpvdXJuYWwgb2YgV2F0ZXIgYW5kIExhbmQgRGV2ZWxvcG1lbnQiLCJpbXBhY3RGYWN0b3IiOiIxLjQ2In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTMwLCJsYXN0UGFnZSI6MTM2fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkRlIEdydXl0ZXIgT3BlbiJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MzIsImNpdGVkQnlDb3VudCI6MX19LCJxdW90ZSI6IkluIHRoaXMgY29udGV4dCwgdGhlIGFkb3B0aW9uIG9mICBzdXN0YWluYWJsZSBpcnJpZ2F0aW9uIHByYWN0aWNlcyBoYXMgZW1lcmdlZCBhcyBhIGNyaXRpY2FsIHN0cmF0ZWd5ICBmb3IgZW5oYW5jaW5nIGZvb2Qgc2VjdXJpdHkgd2hpbGUgcHJvbW90aW5nIHdhdGVyIGNvbnNlcnZhdGlvbiAgaW4gYWdyaWN1bHR1cmFsIGxhbmRzY2FwZXMuICBJcnJpZ2F0aW9uIHBsYXlzIGEgdml0YWwgcm9sZSBpbiBnbG9iYWwgZm9vZCBwcm9kdWN0aW9uLCB3aXRoICBpcnJpZ2F0ZWQgYWdyaWN1bHR1cmUgYWNjb3VudGluZyBmb3IgYXBwcm94aW1hdGVseSA0MCUgb2YgdGhlICBmb29kIHN1cHBseSAoV3JhY2hpZW4gZGUsIFNjaHVsdHogYW5kIEdvbGksIDIwMjE7IFNjaG1pdHQsIFJvc2EgIGFuZCBEYWlseSwgMjAyMikuIEhvd2V2ZXIsIHRyYWRpdGlvbmFsIGlycmlnYXRpb24gbWV0aG9kcywgc3VjaCBhcyAgZmxvb2QgYW5kIGZ1cnJvdyBpcnJpZ2F0aW9uLCBhcmUgb2Z0ZW4gaW5lZmZpY2llbnQsIGxlYWRpbmcgdG8gIHN1YnN0YW50aWFsIHdhdGVyIGxvc3NlcyB0aHJvdWdoIGV2YXBvcmF0aW9uLCBydW5vZmYsIGFuZCBkZWVwICBwZXJjb2xhdGlvbiAoQW1ib21zYSwgMjAyMCkuIE1vcmVvdmVyLCB0aGUgZXhjZXNzaXZlIGFwcGxpY2EtdGlvbiBvZiB3YXRlciBjYW4gcmVzdWx0IGluIHNvaWwgc2FsaW5pc2F0aW9uLCBudXRyaWVudCBsZWFjaGluZywgYW5kICBncm91bmR3YXRlciBkZXBsZXRpb24sIGZ1cnRoZXIgY29tcHJvbWlzaW5nIHRoZSBsb25nLXRlcm0gIHN1c3RhaW5hYmlsaXR5IG9mIGFncmljdWx0dXJhbCBzeXN0ZW1zIChZaW4gZXQgYWwuLCAyMDIyKS4gVG8gYWRkcmVzcyAgdGhlc2UgY2hhbGxlbmdlcywgcmVzZWFyY2hlcnMgYW5kIHByYWN0aXRpb25lcnMgaGF2ZSBiZWVuIGV4cGxvci1pbmcgaW5ub3ZhdGl2ZSBpcnJpZ2F0aW9uIHRlY2huaXF1ZXMgYW5kIHdhdGVyIG1hbmFnZW1lbnQgIHN0cmF0ZWdpZXMgdGhhdCBjYW4gb3B0aW1pc2Ugd2F0ZXIgdXNlIGVmZmljaWVuY3kgKFdVRSkgd2hpbGUgIG1haW50YWluaW5nIG9yIGVuaGFuY2luZyBjcm9wIHByb2R1Y3Rpdml0eS4gIFByZWNpc2lvbiBpcnJpZ2F0aW9uIChQSSkgb2ZmZXJzIGEgcHJvbWlzaW5nIHNvbHV0aW9uLCAgaW52b2x2aW5nIHRoZSB0YXJnZXRlZCBhcHBsaWNhdGlvbiBvZiB3YXRlciBzcGVjaWZpY2FsbHkgdG8gdGhlICBuZWVkcyBvZiBpbmRpdmlkdWFsIHBsYW50cyBvciBjcm9wIHpvbmVzIChaaW5rZXJuYWdlbCBldCBhbC4sIDIwMjApLiAgIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyMi44ODY3MzU5MTYxMzc2OTUsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzQ2MjIxOTIzODI4MTI1LCJoZWlnaHQiOjEuNDUwNTQxMDE5NDM5Njk3M319LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI0LjM1MTEyMzgwOTgxNDQ1MywibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTI3MTYwNjQ0NSwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI1LjcyNDk4NzAzMDAyOTI5NywibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTY1MzA3NjE3MiwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI3LjA5ODg1MDI1MDI0NDE0LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjE2LjQxODE2MTM5MjIxMTkxNCwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI4LjQ3MjcwOTY1NTc2MTcyLCJsZWZ0IjoxMi4zODA4ODAzNTU4MzQ5NjEsIndpZHRoIjozNy4wMTIxODQxNDMwNjY0MDYsImhlaWdodCI6MS4zNjAwMTQ3OTYyNTcwMTl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyOS44NDY1NzI4NzU5NzY1NjIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQwNjQ4NjUxMTIzMDUsImhlaWdodCI6MS4zNjAwMTY3MDM2MDU2NTE5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzEuMjE0MDM2OTQxNTI4MzIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQ0ODU2MjYyMjA3MDMsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzIuNTg3OTAyMDY5MDkxOCwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDYwNTc4OTE4NDU3LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMzLjk2MTc2MTQ3NDYwOTM3NSwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDA0ODQ2MTkxNDA2MjUsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzUuMzM1NjI0Njk0ODI0MjIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzM5NzQ0NTY3ODcxMDk0LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjM2LjcwOTQ4NzkxNTAzOTA2LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjAwMTU5MDcyODc1OTc2NiwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC4wODMzNDczMjA1NTY2NCwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTY1MzA3NjE3MiwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOS40NTcyMTA1NDA3NzE0ODQsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQzMDQ4MDk1NzAzMTI1LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQwLjc0MDU0NzE4MDE3NTc4LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjM0NTQ1NTE2OTY3NzczNCwiaGVpZ2h0IjoxLjQ1MDU0MjgwNzU3OTA0MDV9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Mi4yMDQ5MzMxNjY1MDM5MDYsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMDAzMjUwMTIyMDcwMzEsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDMuNTc4Nzk2Mzg2NzE4NzUsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzM3OTIxMTQyNTc4MTI1LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQ0Ljg2MjEzMzAyNjEyMzA1LCJsZWZ0Ijo5LjA0NzYwMTY5OTgyOTEwMiwid2lkdGgiOjQwLjM0NTMxNDAyNTg3ODkwNiwiaGVpZ2h0IjoxLjQ1MDU0MjgwNzU3OTA0MDV9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ni4zMjY1MTkwMTI0NTExNywibGVmdCI6OS4wNDc2MDE2OTk4MjkxMDIsIndpZHRoIjoyNy45Njc0NDcyODA4ODM3OSwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ny43MDAzODIyMzI2NjYwMTYsImxlZnQiOjEyLjM4MDg3ODQ0ODQ4NjMyOCwid2lkdGgiOjM3LjAzNDc4NjIyNDM2NTIzNCwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0OS4wNzQyNDU0NTI4ODA4NiwibGVmdCI6OS4wNDc2MDE2OTk4MjkxMDIsIndpZHRoIjo0MC4zNDA2NDQ4MzY0MjU3OCwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1MC4zNTc1ODIwOTIyODUxNTYsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzQ0ODUyNDQ3NTA5NzY2LCJoZWlnaHQiOjEuNDUwNTQyODA3NTc5MDQwNX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEzMSJ9fV0%3D;

- Integration potential: Feeds into smart controllers for automated responses.

### Internet of Things in Irrigation

Internet of Things networks interconnect soil sensors, weather stations, and valves for continuous data flow, enabling automated irrigation that minimizes waste and achieves 30-50% WUE improvements over manual methods by responding to real-time conditions (Jumintono et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikp1bWludG9ubyIsImZhbWlseSI6Ikp1bWludG9ubyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJOdXJuYXdhdHkiLCJmYW1pbHkiOiJOdXJuYXdhdHkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTnVncm9obyBUcmlzbnUiLCJmYW1pbHkiOiJCcmF0YSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJEaW5pIiwiZmFtaWx5IjoiUmFobWlhdGkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSGFkaSIsImZhbWlseSI6IlN1c2lsbyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBLiBBLiIsImZhbWlseSI6Ikh1c3NlbiJ9fV0sImlkcyI6eyJpc3NuIjpbIjE0MjktNzQyNiIsIjIwODMtNDUzNSJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMjQ0MjUvandsZC4yMDI0LjE1MTc5OCIsIm9wZW5hbGV4IjoiVzQ0MDU0NzcxMDAifSwidGl0bGUiOiJDb21wYXJhdGl2ZSBhbmFseXNpcyBvZiBwcmVjaXNpb24sIGRlZmljaXQsIGFuZCBkcmlwIGlycmlnYXRpb24gZm9yIHdhdGVyIGVmZmljaWVuY3kgaW4gSXJhcSIsImFic3RyYWN0IjoiU3VzdGFpbmFibGUgaXJyaWdhdGlvbiBwcmFjdGljZXMgYXJlIGNyaXRpY2FsIGZvciBhZGRyZXNzaW5nIHRoZSBjaGFsbGVuZ2VzIG9mIHdhdGVyIHNjYXJjaXR5IGFuZCBmb29kIGluc2VjdXJpdHkgaW4gdGhlIGZhY2Ugb2YgY2xpbWF0ZSBjaGFuZ2UgYW5kIGdyb3dpbmcgZ2xvYmFsIGZvb2QgZGVtYW5kLiBUaGlzIHN0dWR5IGludmVzdGlnYXRlZCB0aGUgcG90ZW50aWFsIG9mIHByZWNpc2lvbiBpcnJpZ2F0aW9uIChQSSksIGRlZmljaXQgaXJyaWdhdGlvbiAoREkpLCBhbmQgZHJpcCBpcnJpZ2F0aW9uIChEUkkpIHRlY2huaXF1ZXMgdG8gZW5oYW5jZSBjcm9wIHByb2R1Y3Rpdml0eSBhbmQgd2F0ZXIgdXNlIGVmZmljaWVuY3kgKFdVRSkgY29tcGFyZWQgdG8gY29udmVudGlvbmFsIGZsb29kIGlycmlnYXRpb24gKENGSSkgaW4gdGhyZWUgZGlzdGluY3QgYWdyb2Vjb2xvZ2ljYWwgem9uZXMgb2YgSXJhcS4gRmllbGQgZXhwZXJpbWVudHMgd2VyZSBjb25kdWN0ZWQgdXNpbmcgYSByYW5kb21pc2VkIGNvbXBsZXRlIGJsb2NrIGRlc2lnbiB3aXRoIHdoZWF0LCBtYWl6ZSwgYW5kIHJpY2UgY3JvcHMuIFRoZSByZXN1bHRzIGRlbW9uc3RyYXRlZCB0aGF0IFBJIHNpZ25pZmljYW50bHkgaW5jcmVhc2VkIGNyb3AgZ3Jvd3RoIHBhcmFtZXRlcnMsIGdyYWluIHlpZWxkLCBhbmQgV1VFIGFjcm9zcyBhbGwgem9uZXMsIHdpdGggeWllbGQgaW1wcm92ZW1lbnRzIG9mIDMz4oCTMzglIGFuZCBXVUUgaW5jcmVhc2VzIG9mIDQ24oCTNTElIGluIGNvbnRyYXN0IHdpdGggQ0ZJLiBUaGUgREkgYW5kIERSSSB0cmVhdG1lbnRzIGFsc28gb3V0cGVyZm9ybWVkIENGSSwgYWxiZWl0IHRvIGEgbGVzc2VyIGV4dGVudC4gUmVtb3RlIHNlbnNpbmctZGVyaXZlZCB2ZWdldGF0aW9uIGluZGljZXMgc3Ryb25nbHkgY29ycmVsYXRlZCB3aXRoIGNyb3AgZ3Jvd3RoIHBhcmFtZXRlcnMgYW5kIHlpZWxkLCB3aGlsZSBoeWRyb2xvZ2ljYWwgbW9kZWxsaW5nIHJldmVhbGVkIHJlZHVjZWQgZXZhcG90cmFuc3BpcmF0aW9uIGFuZCBzdXJmYWNlIHJ1bm9mZiB1bmRlciB0aGUgUEkgdHJlYXRtZW50LiBUaGUgc3VzdGFpbmFibGUgaXJyaWdhdGlvbiBwcmFjdGljZXMgcmVzdWx0ZWQgaW4gc3Vic3RhbnRpYWwgd2F0ZXIgc2F2aW5ncyBvZiAyMOKAkzMwJSBjb21wYXJlZCB0byBDRkkuIFRoZXNlIGZpbmRpbmdzIGhpZ2hsaWdodCB0aGUgaW1wb3J0YW5jZSBvZiBhZG9wdGluZyBlZmZpY2llbnQgaXJyaWdhdGlvbiB0ZWNobmlxdWVzLCBhbG9uZyB3aXRoIGEgaG9saXN0aWMgYXBwcm9hY2ggZW5jb21wYXNzaW5nIHRlY2hub2xvZ2ljYWwgaW5ub3ZhdGlvbnMsIGNhcGFjaXR5IGJ1aWxkaW5nLCBhbmQgc3Rha2Vob2xkZXIgZW5nYWdlbWVudCwgdG8gcHJvbW90ZSB3YXRlci0gZWZmaWNpZW50IGFncmljdWx0dXJlIGFuZCBlbnN1cmUgZm9vZCBzZWN1cml0eSBpbiB3YXRlci1zY2FyY2UgcmVnaW9ucy4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjQ0MjUvandsZC4yMDI0LjE1MTc5OCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjI0NDI1L2p3bGQuMjAyNC4xNTE3OTgiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMi0xNCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkpvdXJuYWwgb2YgV2F0ZXIgYW5kIExhbmQgRGV2ZWxvcG1lbnQiLCJpbXBhY3RGYWN0b3IiOiIxLjQ2In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTMwLCJsYXN0UGFnZSI6MTM2fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkRlIEdydXl0ZXIgT3BlbiJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MzIsImNpdGVkQnlDb3VudCI6MX19LCJxdW90ZSI6IkluIHRoaXMgY29udGV4dCwgdGhlIGFkb3B0aW9uIG9mICBzdXN0YWluYWJsZSBpcnJpZ2F0aW9uIHByYWN0aWNlcyBoYXMgZW1lcmdlZCBhcyBhIGNyaXRpY2FsIHN0cmF0ZWd5ICBmb3IgZW5oYW5jaW5nIGZvb2Qgc2VjdXJpdHkgd2hpbGUgcHJvbW90aW5nIHdhdGVyIGNvbnNlcnZhdGlvbiAgaW4gYWdyaWN1bHR1cmFsIGxhbmRzY2FwZXMuICBJcnJpZ2F0aW9uIHBsYXlzIGEgdml0YWwgcm9sZSBpbiBnbG9iYWwgZm9vZCBwcm9kdWN0aW9uLCB3aXRoICBpcnJpZ2F0ZWQgYWdyaWN1bHR1cmUgYWNjb3VudGluZyBmb3IgYXBwcm94aW1hdGVseSA0MCUgb2YgdGhlICBmb29kIHN1cHBseSAoV3JhY2hpZW4gZGUsIFNjaHVsdHogYW5kIEdvbGksIDIwMjE7IFNjaG1pdHQsIFJvc2EgIGFuZCBEYWlseSwgMjAyMikuIEhvd2V2ZXIsIHRyYWRpdGlvbmFsIGlycmlnYXRpb24gbWV0aG9kcywgc3VjaCBhcyAgZmxvb2QgYW5kIGZ1cnJvdyBpcnJpZ2F0aW9uLCBhcmUgb2Z0ZW4gaW5lZmZpY2llbnQsIGxlYWRpbmcgdG8gIHN1YnN0YW50aWFsIHdhdGVyIGxvc3NlcyB0aHJvdWdoIGV2YXBvcmF0aW9uLCBydW5vZmYsIGFuZCBkZWVwICBwZXJjb2xhdGlvbiAoQW1ib21zYSwgMjAyMCkuIE1vcmVvdmVyLCB0aGUgZXhjZXNzaXZlIGFwcGxpY2EtdGlvbiBvZiB3YXRlciBjYW4gcmVzdWx0IGluIHNvaWwgc2FsaW5pc2F0aW9uLCBudXRyaWVudCBsZWFjaGluZywgYW5kICBncm91bmR3YXRlciBkZXBsZXRpb24sIGZ1cnRoZXIgY29tcHJvbWlzaW5nIHRoZSBsb25nLXRlcm0gIHN1c3RhaW5hYmlsaXR5IG9mIGFncmljdWx0dXJhbCBzeXN0ZW1zIChZaW4gZXQgYWwuLCAyMDIyKS4gVG8gYWRkcmVzcyAgdGhlc2UgY2hhbGxlbmdlcywgcmVzZWFyY2hlcnMgYW5kIHByYWN0aXRpb25lcnMgaGF2ZSBiZWVuIGV4cGxvci1pbmcgaW5ub3ZhdGl2ZSBpcnJpZ2F0aW9uIHRlY2huaXF1ZXMgYW5kIHdhdGVyIG1hbmFnZW1lbnQgIHN0cmF0ZWdpZXMgdGhhdCBjYW4gb3B0aW1pc2Ugd2F0ZXIgdXNlIGVmZmljaWVuY3kgKFdVRSkgd2hpbGUgIG1haW50YWluaW5nIG9yIGVuaGFuY2luZyBjcm9wIHByb2R1Y3Rpdml0eS4gIFByZWNpc2lvbiBpcnJpZ2F0aW9uIChQSSkgb2ZmZXJzIGEgcHJvbWlzaW5nIHNvbHV0aW9uLCAgaW52b2x2aW5nIHRoZSB0YXJnZXRlZCBhcHBsaWNhdGlvbiBvZiB3YXRlciBzcGVjaWZpY2FsbHkgdG8gdGhlICBuZWVkcyBvZiBpbmRpdmlkdWFsIHBsYW50cyBvciBjcm9wIHpvbmVzIChaaW5rZXJuYWdlbCBldCBhbC4sIDIwMjApLiAgIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyMi44ODY3MzU5MTYxMzc2OTUsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzQ2MjIxOTIzODI4MTI1LCJoZWlnaHQiOjEuNDUwNTQxMDE5NDM5Njk3M319LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI0LjM1MTEyMzgwOTgxNDQ1MywibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTI3MTYwNjQ0NSwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI1LjcyNDk4NzAzMDAyOTI5NywibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTY1MzA3NjE3MiwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI3LjA5ODg1MDI1MDI0NDE0LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjE2LjQxODE2MTM5MjIxMTkxNCwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI4LjQ3MjcwOTY1NTc2MTcyLCJsZWZ0IjoxMi4zODA4ODAzNTU4MzQ5NjEsIndpZHRoIjozNy4wMTIxODQxNDMwNjY0MDYsImhlaWdodCI6MS4zNjAwMTQ3OTYyNTcwMTl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyOS44NDY1NzI4NzU5NzY1NjIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQwNjQ4NjUxMTIzMDUsImhlaWdodCI6MS4zNjAwMTY3MDM2MDU2NTE5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzEuMjE0MDM2OTQxNTI4MzIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQ0ODU2MjYyMjA3MDMsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzIuNTg3OTAyMDY5MDkxOCwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDYwNTc4OTE4NDU3LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMzLjk2MTc2MTQ3NDYwOTM3NSwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDA0ODQ2MTkxNDA2MjUsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzUuMzM1NjI0Njk0ODI0MjIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzM5NzQ0NTY3ODcxMDk0LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjM2LjcwOTQ4NzkxNTAzOTA2LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjAwMTU5MDcyODc1OTc2NiwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC4wODMzNDczMjA1NTY2NCwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTY1MzA3NjE3MiwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOS40NTcyMTA1NDA3NzE0ODQsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQzMDQ4MDk1NzAzMTI1LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQwLjc0MDU0NzE4MDE3NTc4LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjM0NTQ1NTE2OTY3NzczNCwiaGVpZ2h0IjoxLjQ1MDU0MjgwNzU3OTA0MDV9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Mi4yMDQ5MzMxNjY1MDM5MDYsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMDAzMjUwMTIyMDcwMzEsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDMuNTc4Nzk2Mzg2NzE4NzUsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzM3OTIxMTQyNTc4MTI1LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQ0Ljg2MjEzMzAyNjEyMzA1LCJsZWZ0Ijo5LjA0NzYwMTY5OTgyOTEwMiwid2lkdGgiOjQwLjM0NTMxNDAyNTg3ODkwNiwiaGVpZ2h0IjoxLjQ1MDU0MjgwNzU3OTA0MDV9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ni4zMjY1MTkwMTI0NTExNywibGVmdCI6OS4wNDc2MDE2OTk4MjkxMDIsIndpZHRoIjoyNy45Njc0NDcyODA4ODM3OSwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ny43MDAzODIyMzI2NjYwMTYsImxlZnQiOjEyLjM4MDg3ODQ0ODQ4NjMyOCwid2lkdGgiOjM3LjAzNDc4NjIyNDM2NTIzNCwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0OS4wNzQyNDU0NTI4ODA4NiwibGVmdCI6OS4wNDc2MDE2OTk4MjkxMDIsIndpZHRoIjo0MC4zNDA2NDQ4MzY0MjU3OCwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1MC4zNTc1ODIwOTIyODUxNTYsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzQ0ODUyNDQ3NTA5NzY2LCJoZWlnaHQiOjEuNDUwNTQyODA3NTc5MDQwNX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEzMSJ9fV0%3D. This connectivity transforms static systems into adaptive ones, addressing inefficiencies in traditional practices that contribute to resource overuse (Daraz et al., 2025)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlVtYXIiLCJmYW1pbHkiOiJEYXJheiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiLFoHRlZmFuIiwiZmFtaWx5IjoiQm9qbmVjIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllvdW5hcyIsImZhbWlseSI6IktoYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMDc3LTA0NzIiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzOTAvYWdyaWN1bHR1cmUxNTA1MDU1NCIsIm9wZW5hbGV4IjoiVzQ0MDgyMTk4MTEiLCJleGEiOiJodHRwczovL3d3dy5tZHBpLmNvbS8yMDc3LTA0NzIvMTUvNS81NTQifSwidGl0bGUiOiJFbmVyZ3ktRWZmaWNpZW50IFNtYXJ0IElycmlnYXRpb24gVGVjaG5vbG9naWVzOiBBIFBhdGh3YXkgdG8gV2F0ZXIgYW5kIEVuZXJneSBTdXN0YWluYWJpbGl0eSBpbiBBZ3JpY3VsdHVyZSIsImFic3RyYWN0IjoiVGhlIGFncmljdWx0dXJhbCBzZWN0b3IgZmFjZXMgY2hhbGxlbmdlcyBzdWNoIGFzIHdhdGVyIHNjYXJjaXR5LCBlbmVyZ3kgaW5lZmZpY2llbmN5LCBhbmQgZGVjbGluaW5nIHByb2R1Y3Rpdml0eSwgcGFydGljdWxhcmx5IGluIGFyaWQgcmVnaW9ucy4gVHJhZGl0aW9uYWwgaXJyaWdhdGlvbiBtZXRob2RzIGNvbnRyaWJ1dGUgdG8gcmVzb3VyY2UgZGVwbGV0aW9uIGFuZCBlbnZpcm9ubWVudGFsIGltcGFjdHMuIFNvbGFyLXBvd2VyZWQgc21hcnQgaXJyaWdhdGlvbiBzeXN0ZW1zIGludGVncmF0ZSBwcmVjaXNpb24gaXJyaWdhdGlvbiB3aXRoIHJlbmV3YWJsZSBlbmVyZ3ksIGltcHJvdmluZyB3YXRlciB1c2UgYW5kIHByb2R1Y3Rpdml0eS4gSW4gUGFraXN0YW4sIHdoZXJlIGFncmljdWx0dXJlIGNvbnRyaWJ1dGVzIDE5JSBvZiBncm9zcyBkb21lc3RpYyBwcm9kdWN0IGFuZCBlbXBsb3lzIDQwJSBvZiB0aGUgd29ya2ZvcmNlLCB0aGVzZSBjaGFsbGVuZ2VzIGFyZSBzZXZlcmUsIGVzcGVjaWFsbHkgaW4gd2F0ZXItc2NhcmNlIGFyZWFzIGxpa2UgdGhlIENob2xpc3RhbiBEZXNlcnQuIFRoaXMgc3R1ZHkgZXhhbWluZXMgdGhlIGltcGFjdCBvZiBzb2xhci1wb3dlcmVkIHNtYXJ0IGlycmlnYXRpb24gb24gYWdyaWN1bHR1cmFsIHByb2R1Y3Rpdml0eSwgd2F0ZXIgY29uc2VydmF0aW9uLCBhbmQgZW5lcmd5IGVmZmljaWVuY3kgaW4gdGhlIENob2xpc3RhbiBEZXNlcnQuIFVzaW5nIGEgcXVhbnRpdGF0aXZlIGNyb3NzLXNlY3Rpb25hbCBkZXNpZ24sIGRhdGEgd2VyZSBjb2xsZWN0ZWQgZnJvbSAzODQgZmFybWVycyB2aWEgc3RydWN0dXJlZCBxdWVzdGlvbm5haXJlcy4gU3RhdGlzdGljYWwgYW5hbHlzZXMsIGluY2x1ZGluZyBtdWx0aXBsZSBsaW5lYXIgcmVncmVzc2lvbiwgcGFpcmVkIHNhbXBsZSB0LXRlc3RzLCBhbmQgU3RydWN0dXJhbCBFcXVhdGlvbiBNb2RlbGluZyAoU0VNKSwgd2VyZSBjb25kdWN0ZWQuIEZpbmRpbmdzIHNob3cgc2lnbmlmaWNhbnQgaW1wcm92ZW1lbnRzIGluIGNyb3AgeWllbGQgKGZyb20gMy4wIHRvIDQuOCB0b25zL2hlY3RhcmUpIGFuZCByZWR1Y3Rpb25zIGluIHdhdGVyIGFuZCBlbmVyZ3kgY29uc3VtcHRpb24uIFJlZ3Jlc3Npb24gYW5hbHlzaXMgaGlnaGxpZ2h0ZWQgc3Ryb25nIHBvc2l0aXZlIGVmZmVjdHMgb24geWllbGQgYW5kIGVmZmljaWVuY3ksIHdoaWxlIFNFTSBjb25maXJtZWQgcmVkdWNlZCBlbnZpcm9ubWVudGFsIGltcGFjdCBhbmQgb3BlcmF0aW9uYWwgY29zdHMuIFRoZSBzdHVkeSBjb25jbHVkZXMgdGhhdCBzb2xhci1wb3dlcmVkIGlycmlnYXRpb24gZW5oYW5jZXMgcHJvZHVjdGl2aXR5LCBjb25zZXJ2ZXMgcmVzb3VyY2VzLCBhbmQgcHJvbW90ZXMgc3VzdGFpbmFiaWxpdHkuIFBvbGljeW1ha2VycyBzaG91bGQgcHJvdmlkZSBmaW5hbmNpYWwgaW5jZW50aXZlcywgaW52ZXN0IGluIHJlbmV3YWJsZSBpbmZyYXN0cnVjdHVyZSwgYW5kIGltcGxlbWVudCB0cmFpbmluZyBwcm9ncmFtcyB0byBzdXBwb3J0IGFkb3B0aW9uLiBDb2xsYWJvcmF0aXZlIGVmZm9ydHMgYXJlIGVzc2VudGlhbCBmb3Igc3VzdGFpbmFibGUgYWdyaWN1bHR1cmUgaW4gYXJpZCByZWdpb25zLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJpc3N1ZWREYXRlIjoiMjAyNS0wMy0wNSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkFncmljdWx0dXJlIiwiaXNzdWUiOjUsInZvbHVtZSI6MTUsImltcGFjdEZhY3RvciI6IjUuMjYifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjo1NTQsImxhc3RQYWdlIjo1NTR9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiTXVsdGlkaXNjaXBsaW5hcnkgRGlnaXRhbCBQdWJsaXNoaW5nIEluc3RpdHV0ZSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NDgsImNpdGVkQnlDb3VudCI6Mn19LCJxdW90ZSI6IjEuIEludHJvZHVjdGlvbiBUaGUgYWdyaWN1bHR1cmFsIHNlY3RvciwgYSBjb3JuZXJzdG9uZSBvZiBnbG9iYWwgZm9vZCBzZWN1cml0eSBhbmQgZWNvbm9taWMgZGV2ZWxvcG1lbnQsIGZhY2VzIG1vdW50aW5nIGNoYWxsZW5nZXMgaW4gdGhlIDIxc3QgY2VudHVyeS4gQ2xpbWF0ZSBjaGFuZ2UsIHJhcGlkIHBvcHVsYXRpb24gZ3Jvd3RoLCBhbmQgZHdpbmRsaW5nIG5hdHVyYWwgcmVzb3VyY2VzIGhhdmUgaW50ZW5zaWZpZWQgdGhlIG5lZWQgZm9yIHN1c3RhaW5hYmxlIGFncmljdWx0dXJhbCBwcmFjdGljZXMuIEFtb25nIHRoZXNlIGNoYWxsZW5nZXMsIHdhdGVyIHNjYXJjaXR5IGFuZCBlbmVyZ3kgaW5lZmZpY2llbmN5IHN0YW5kIG91dCBhcyBjcml0aWNhbCBiYXJyaWVycyB0byBwcm9kdWN0aXZpdHkgYW5kIHN1c3RhaW5hYmlsaXR5LiBHbG9iYWxseSwgYWdyaWN1bHR1cmUgYWNjb3VudHMgZm9yIGFwcHJveGltYXRlbHkgNzAlIG9mIGZyZXNod2F0ZXIgd2l0aGRyYXdhbHMgYW5kIGlzIG9uZSBvZiB0aGUgbGFyZ2VzdCBjb25zdW1lcnMgb2YgZW5lcmd5IGluIG1hbnkgcmVnaW9ucyBbIDEsIDJdLiBUcmFkaXRpb25hbCBpcnJpZ2F0aW9uIHByYWN0aWNlcywgb2Z0ZW4gaW5lZmZpY2llbnQgYW5kIHJlc291cmNlLWludGVuc2l2ZSwgZXhhY2VyYmF0ZSB0aGVzZSBpc3N1ZXMsIGxlYWRpbmcgdG8gd2F0ZXIgd2FzdGFnZSwgZW5lcmd5IG92ZXJ1c2UsIGFuZCByZWR1Y2VkIGNyb3AgeWllbGRzLiJ9XQ%3D%3D.

### Table 7: Benefits of Precision Irrigation Adoption

| Benefit Category | Expected Outcome | Specific Metric/Impact |
| --- | --- | --- |
| Water Savings | 50–90% reduction | Lower withdrawal from sources |
| Energy Efficiency | Slashed demands | Lower carbon footprint |
| Yield Increase | Elevated productivity | More food per unit of water |
| Economic Profit | Lower input costs | Higher net returns for farmers |
| Environmental | Soil health preservation | Reduced salinization/erosion |
| Adaptive Capacity | Resilience to drought | Stable production in arid zones |
| Resource Mgmt | Targeted delivery | Individual plant monitoring |
| Sustainability | Long-term viability | Securing global food security |

Principles of Operation

Low-power wide-area networks like LoRa transmit sensor data (e.g., tensiometers, hygrometers) to edge devices or cloud platforms, where algorithms trigger actuators based on thresholds or models (Daraz et al., 2025)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlVtYXIiLCJmYW1pbHkiOiJEYXJheiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiLFoHRlZmFuIiwiZmFtaWx5IjoiQm9qbmVjIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllvdW5hcyIsImZhbWlseSI6IktoYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMDc3LTA0NzIiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzOTAvYWdyaWN1bHR1cmUxNTA1MDU1NCIsIm9wZW5hbGV4IjoiVzQ0MDgyMTk4MTEiLCJleGEiOiJodHRwczovL3d3dy5tZHBpLmNvbS8yMDc3LTA0NzIvMTUvNS81NTQifSwidGl0bGUiOiJFbmVyZ3ktRWZmaWNpZW50IFNtYXJ0IElycmlnYXRpb24gVGVjaG5vbG9naWVzOiBBIFBhdGh3YXkgdG8gV2F0ZXIgYW5kIEVuZXJneSBTdXN0YWluYWJpbGl0eSBpbiBBZ3JpY3VsdHVyZSIsImFic3RyYWN0IjoiVGhlIGFncmljdWx0dXJhbCBzZWN0b3IgZmFjZXMgY2hhbGxlbmdlcyBzdWNoIGFzIHdhdGVyIHNjYXJjaXR5LCBlbmVyZ3kgaW5lZmZpY2llbmN5LCBhbmQgZGVjbGluaW5nIHByb2R1Y3Rpdml0eSwgcGFydGljdWxhcmx5IGluIGFyaWQgcmVnaW9ucy4gVHJhZGl0aW9uYWwgaXJyaWdhdGlvbiBtZXRob2RzIGNvbnRyaWJ1dGUgdG8gcmVzb3VyY2UgZGVwbGV0aW9uIGFuZCBlbnZpcm9ubWVudGFsIGltcGFjdHMuIFNvbGFyLXBvd2VyZWQgc21hcnQgaXJyaWdhdGlvbiBzeXN0ZW1zIGludGVncmF0ZSBwcmVjaXNpb24gaXJyaWdhdGlvbiB3aXRoIHJlbmV3YWJsZSBlbmVyZ3ksIGltcHJvdmluZyB3YXRlciB1c2UgYW5kIHByb2R1Y3Rpdml0eS4gSW4gUGFraXN0YW4sIHdoZXJlIGFncmljdWx0dXJlIGNvbnRyaWJ1dGVzIDE5JSBvZiBncm9zcyBkb21lc3RpYyBwcm9kdWN0IGFuZCBlbXBsb3lzIDQwJSBvZiB0aGUgd29ya2ZvcmNlLCB0aGVzZSBjaGFsbGVuZ2VzIGFyZSBzZXZlcmUsIGVzcGVjaWFsbHkgaW4gd2F0ZXItc2NhcmNlIGFyZWFzIGxpa2UgdGhlIENob2xpc3RhbiBEZXNlcnQuIFRoaXMgc3R1ZHkgZXhhbWluZXMgdGhlIGltcGFjdCBvZiBzb2xhci1wb3dlcmVkIHNtYXJ0IGlycmlnYXRpb24gb24gYWdyaWN1bHR1cmFsIHByb2R1Y3Rpdml0eSwgd2F0ZXIgY29uc2VydmF0aW9uLCBhbmQgZW5lcmd5IGVmZmljaWVuY3kgaW4gdGhlIENob2xpc3RhbiBEZXNlcnQuIFVzaW5nIGEgcXVhbnRpdGF0aXZlIGNyb3NzLXNlY3Rpb25hbCBkZXNpZ24sIGRhdGEgd2VyZSBjb2xsZWN0ZWQgZnJvbSAzODQgZmFybWVycyB2aWEgc3RydWN0dXJlZCBxdWVzdGlvbm5haXJlcy4gU3RhdGlzdGljYWwgYW5hbHlzZXMsIGluY2x1ZGluZyBtdWx0aXBsZSBsaW5lYXIgcmVncmVzc2lvbiwgcGFpcmVkIHNhbXBsZSB0LXRlc3RzLCBhbmQgU3RydWN0dXJhbCBFcXVhdGlvbiBNb2RlbGluZyAoU0VNKSwgd2VyZSBjb25kdWN0ZWQuIEZpbmRpbmdzIHNob3cgc2lnbmlmaWNhbnQgaW1wcm92ZW1lbnRzIGluIGNyb3AgeWllbGQgKGZyb20gMy4wIHRvIDQuOCB0b25zL2hlY3RhcmUpIGFuZCByZWR1Y3Rpb25zIGluIHdhdGVyIGFuZCBlbmVyZ3kgY29uc3VtcHRpb24uIFJlZ3Jlc3Npb24gYW5hbHlzaXMgaGlnaGxpZ2h0ZWQgc3Ryb25nIHBvc2l0aXZlIGVmZmVjdHMgb24geWllbGQgYW5kIGVmZmljaWVuY3ksIHdoaWxlIFNFTSBjb25maXJtZWQgcmVkdWNlZCBlbnZpcm9ubWVudGFsIGltcGFjdCBhbmQgb3BlcmF0aW9uYWwgY29zdHMuIFRoZSBzdHVkeSBjb25jbHVkZXMgdGhhdCBzb2xhci1wb3dlcmVkIGlycmlnYXRpb24gZW5oYW5jZXMgcHJvZHVjdGl2aXR5LCBjb25zZXJ2ZXMgcmVzb3VyY2VzLCBhbmQgcHJvbW90ZXMgc3VzdGFpbmFiaWxpdHkuIFBvbGljeW1ha2VycyBzaG91bGQgcHJvdmlkZSBmaW5hbmNpYWwgaW5jZW50aXZlcywgaW52ZXN0IGluIHJlbmV3YWJsZSBpbmZyYXN0cnVjdHVyZSwgYW5kIGltcGxlbWVudCB0cmFpbmluZyBwcm9ncmFtcyB0byBzdXBwb3J0IGFkb3B0aW9uLiBDb2xsYWJvcmF0aXZlIGVmZm9ydHMgYXJlIGVzc2VudGlhbCBmb3Igc3VzdGFpbmFibGUgYWdyaWN1bHR1cmUgaW4gYXJpZCByZWdpb25zLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJpc3N1ZWREYXRlIjoiMjAyNS0wMy0wNSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkFncmljdWx0dXJlIiwiaXNzdWUiOjUsInZvbHVtZSI6MTUsImltcGFjdEZhY3RvciI6IjUuMjYifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjo1NTQsImxhc3RQYWdlIjo1NTR9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiTXVsdGlkaXNjaXBsaW5hcnkgRGlnaXRhbCBQdWJsaXNoaW5nIEluc3RpdHV0ZSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NDgsImNpdGVkQnlDb3VudCI6Mn19LCJxdW90ZSI6IjEuIEludHJvZHVjdGlvbiBUaGUgYWdyaWN1bHR1cmFsIHNlY3RvciwgYSBjb3JuZXJzdG9uZSBvZiBnbG9iYWwgZm9vZCBzZWN1cml0eSBhbmQgZWNvbm9taWMgZGV2ZWxvcG1lbnQsIGZhY2VzIG1vdW50aW5nIGNoYWxsZW5nZXMgaW4gdGhlIDIxc3QgY2VudHVyeS4gQ2xpbWF0ZSBjaGFuZ2UsIHJhcGlkIHBvcHVsYXRpb24gZ3Jvd3RoLCBhbmQgZHdpbmRsaW5nIG5hdHVyYWwgcmVzb3VyY2VzIGhhdmUgaW50ZW5zaWZpZWQgdGhlIG5lZWQgZm9yIHN1c3RhaW5hYmxlIGFncmljdWx0dXJhbCBwcmFjdGljZXMuIEFtb25nIHRoZXNlIGNoYWxsZW5nZXMsIHdhdGVyIHNjYXJjaXR5IGFuZCBlbmVyZ3kgaW5lZmZpY2llbmN5IHN0YW5kIG91dCBhcyBjcml0aWNhbCBiYXJyaWVycyB0byBwcm9kdWN0aXZpdHkgYW5kIHN1c3RhaW5hYmlsaXR5LiBHbG9iYWxseSwgYWdyaWN1bHR1cmUgYWNjb3VudHMgZm9yIGFwcHJveGltYXRlbHkgNzAlIG9mIGZyZXNod2F0ZXIgd2l0aGRyYXdhbHMgYW5kIGlzIG9uZSBvZiB0aGUgbGFyZ2VzdCBjb25zdW1lcnMgb2YgZW5lcmd5IGluIG1hbnkgcmVnaW9ucyBbIDEsIDJdLiBUcmFkaXRpb25hbCBpcnJpZ2F0aW9uIHByYWN0aWNlcywgb2Z0ZW4gaW5lZmZpY2llbnQgYW5kIHJlc291cmNlLWludGVuc2l2ZSwgZXhhY2VyYmF0ZSB0aGVzZSBpc3N1ZXMsIGxlYWRpbmcgdG8gd2F0ZXIgd2FzdGFnZSwgZW5lcmd5IG92ZXJ1c2UsIGFuZCByZWR1Y2VkIGNyb3AgeWllbGRzLiJ9XQ%3D%3D.

Advantages and Efficiency Gains

- Real-time responsiveness: Prevents deficits/excess, cutting energy use (Daraz et al., 2025)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlVtYXIiLCJmYW1pbHkiOiJEYXJheiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiLFoHRlZmFuIiwiZmFtaWx5IjoiQm9qbmVjIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllvdW5hcyIsImZhbWlseSI6IktoYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMDc3LTA0NzIiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzOTAvYWdyaWN1bHR1cmUxNTA1MDU1NCIsIm9wZW5hbGV4IjoiVzQ0MDgyMTk4MTEiLCJleGEiOiJodHRwczovL3d3dy5tZHBpLmNvbS8yMDc3LTA0NzIvMTUvNS81NTQifSwidGl0bGUiOiJFbmVyZ3ktRWZmaWNpZW50IFNtYXJ0IElycmlnYXRpb24gVGVjaG5vbG9naWVzOiBBIFBhdGh3YXkgdG8gV2F0ZXIgYW5kIEVuZXJneSBTdXN0YWluYWJpbGl0eSBpbiBBZ3JpY3VsdHVyZSIsImFic3RyYWN0IjoiVGhlIGFncmljdWx0dXJhbCBzZWN0b3IgZmFjZXMgY2hhbGxlbmdlcyBzdWNoIGFzIHdhdGVyIHNjYXJjaXR5LCBlbmVyZ3kgaW5lZmZpY2llbmN5LCBhbmQgZGVjbGluaW5nIHByb2R1Y3Rpdml0eSwgcGFydGljdWxhcmx5IGluIGFyaWQgcmVnaW9ucy4gVHJhZGl0aW9uYWwgaXJyaWdhdGlvbiBtZXRob2RzIGNvbnRyaWJ1dGUgdG8gcmVzb3VyY2UgZGVwbGV0aW9uIGFuZCBlbnZpcm9ubWVudGFsIGltcGFjdHMuIFNvbGFyLXBvd2VyZWQgc21hcnQgaXJyaWdhdGlvbiBzeXN0ZW1zIGludGVncmF0ZSBwcmVjaXNpb24gaXJyaWdhdGlvbiB3aXRoIHJlbmV3YWJsZSBlbmVyZ3ksIGltcHJvdmluZyB3YXRlciB1c2UgYW5kIHByb2R1Y3Rpdml0eS4gSW4gUGFraXN0YW4sIHdoZXJlIGFncmljdWx0dXJlIGNvbnRyaWJ1dGVzIDE5JSBvZiBncm9zcyBkb21lc3RpYyBwcm9kdWN0IGFuZCBlbXBsb3lzIDQwJSBvZiB0aGUgd29ya2ZvcmNlLCB0aGVzZSBjaGFsbGVuZ2VzIGFyZSBzZXZlcmUsIGVzcGVjaWFsbHkgaW4gd2F0ZXItc2NhcmNlIGFyZWFzIGxpa2UgdGhlIENob2xpc3RhbiBEZXNlcnQuIFRoaXMgc3R1ZHkgZXhhbWluZXMgdGhlIGltcGFjdCBvZiBzb2xhci1wb3dlcmVkIHNtYXJ0IGlycmlnYXRpb24gb24gYWdyaWN1bHR1cmFsIHByb2R1Y3Rpdml0eSwgd2F0ZXIgY29uc2VydmF0aW9uLCBhbmQgZW5lcmd5IGVmZmljaWVuY3kgaW4gdGhlIENob2xpc3RhbiBEZXNlcnQuIFVzaW5nIGEgcXVhbnRpdGF0aXZlIGNyb3NzLXNlY3Rpb25hbCBkZXNpZ24sIGRhdGEgd2VyZSBjb2xsZWN0ZWQgZnJvbSAzODQgZmFybWVycyB2aWEgc3RydWN0dXJlZCBxdWVzdGlvbm5haXJlcy4gU3RhdGlzdGljYWwgYW5hbHlzZXMsIGluY2x1ZGluZyBtdWx0aXBsZSBsaW5lYXIgcmVncmVzc2lvbiwgcGFpcmVkIHNhbXBsZSB0LXRlc3RzLCBhbmQgU3RydWN0dXJhbCBFcXVhdGlvbiBNb2RlbGluZyAoU0VNKSwgd2VyZSBjb25kdWN0ZWQuIEZpbmRpbmdzIHNob3cgc2lnbmlmaWNhbnQgaW1wcm92ZW1lbnRzIGluIGNyb3AgeWllbGQgKGZyb20gMy4wIHRvIDQuOCB0b25zL2hlY3RhcmUpIGFuZCByZWR1Y3Rpb25zIGluIHdhdGVyIGFuZCBlbmVyZ3kgY29uc3VtcHRpb24uIFJlZ3Jlc3Npb24gYW5hbHlzaXMgaGlnaGxpZ2h0ZWQgc3Ryb25nIHBvc2l0aXZlIGVmZmVjdHMgb24geWllbGQgYW5kIGVmZmljaWVuY3ksIHdoaWxlIFNFTSBjb25maXJtZWQgcmVkdWNlZCBlbnZpcm9ubWVudGFsIGltcGFjdCBhbmQgb3BlcmF0aW9uYWwgY29zdHMuIFRoZSBzdHVkeSBjb25jbHVkZXMgdGhhdCBzb2xhci1wb3dlcmVkIGlycmlnYXRpb24gZW5oYW5jZXMgcHJvZHVjdGl2aXR5LCBjb25zZXJ2ZXMgcmVzb3VyY2VzLCBhbmQgcHJvbW90ZXMgc3VzdGFpbmFiaWxpdHkuIFBvbGljeW1ha2VycyBzaG91bGQgcHJvdmlkZSBmaW5hbmNpYWwgaW5jZW50aXZlcywgaW52ZXN0IGluIHJlbmV3YWJsZSBpbmZyYXN0cnVjdHVyZSwgYW5kIGltcGxlbWVudCB0cmFpbmluZyBwcm9ncmFtcyB0byBzdXBwb3J0IGFkb3B0aW9uLiBDb2xsYWJvcmF0aXZlIGVmZm9ydHMgYXJlIGVzc2VudGlhbCBmb3Igc3VzdGFpbmFibGUgYWdyaWN1bHR1cmUgaW4gYXJpZCByZWdpb25zLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJpc3N1ZWREYXRlIjoiMjAyNS0wMy0wNSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkFncmljdWx0dXJlIiwiaXNzdWUiOjUsInZvbHVtZSI6MTUsImltcGFjdEZhY3RvciI6IjUuMjYifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjo1NTQsImxhc3RQYWdlIjo1NTR9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiTXVsdGlkaXNjaXBsaW5hcnkgRGlnaXRhbCBQdWJsaXNoaW5nIEluc3RpdHV0ZSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NDgsImNpdGVkQnlDb3VudCI6Mn19LCJxdW90ZSI6IjEuIEludHJvZHVjdGlvbiBUaGUgYWdyaWN1bHR1cmFsIHNlY3RvciwgYSBjb3JuZXJzdG9uZSBvZiBnbG9iYWwgZm9vZCBzZWN1cml0eSBhbmQgZWNvbm9taWMgZGV2ZWxvcG1lbnQsIGZhY2VzIG1vdW50aW5nIGNoYWxsZW5nZXMgaW4gdGhlIDIxc3QgY2VudHVyeS4gQ2xpbWF0ZSBjaGFuZ2UsIHJhcGlkIHBvcHVsYXRpb24gZ3Jvd3RoLCBhbmQgZHdpbmRsaW5nIG5hdHVyYWwgcmVzb3VyY2VzIGhhdmUgaW50ZW5zaWZpZWQgdGhlIG5lZWQgZm9yIHN1c3RhaW5hYmxlIGFncmljdWx0dXJhbCBwcmFjdGljZXMuIEFtb25nIHRoZXNlIGNoYWxsZW5nZXMsIHdhdGVyIHNjYXJjaXR5IGFuZCBlbmVyZ3kgaW5lZmZpY2llbmN5IHN0YW5kIG91dCBhcyBjcml0aWNhbCBiYXJyaWVycyB0byBwcm9kdWN0aXZpdHkgYW5kIHN1c3RhaW5hYmlsaXR5LiBHbG9iYWxseSwgYWdyaWN1bHR1cmUgYWNjb3VudHMgZm9yIGFwcHJveGltYXRlbHkgNzAlIG9mIGZyZXNod2F0ZXIgd2l0aGRyYXdhbHMgYW5kIGlzIG9uZSBvZiB0aGUgbGFyZ2VzdCBjb25zdW1lcnMgb2YgZW5lcmd5IGluIG1hbnkgcmVnaW9ucyBbIDEsIDJdLiBUcmFkaXRpb25hbCBpcnJpZ2F0aW9uIHByYWN0aWNlcywgb2Z0ZW4gaW5lZmZpY2llbnQgYW5kIHJlc291cmNlLWludGVuc2l2ZSwgZXhhY2VyYmF0ZSB0aGVzZSBpc3N1ZXMsIGxlYWRpbmcgdG8gd2F0ZXIgd2FzdGFnZSwgZW5lcmd5IG92ZXJ1c2UsIGFuZCByZWR1Y2VkIGNyb3AgeWllbGRzLiJ9XQ%3D%3D;

- Remote management: Mobile apps allow oversight, reducing labor(Kushwaha et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjMwLCJzdWJ0eXBlIjp7fSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTWFuaXNoIiwiZmFtaWx5IjoiS3VzaHdhaGEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2hhbmthciIsImZhbWlseSI6IlNpbmdoIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlZpamF5IEthbnQiLCJmYW1pbHkiOiJTaW5naCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTaGFzaGFuayIsImZhbWlseSI6IkR3aXZlZGkifX1dLCJpZHMiOnsiaXNzbiI6WyIyNDU2LTE4NzgiXSwiaXNibiI6W10sImRvaSI6IjEwLjIyMTYxL2lqZWFiLjkyLjI3Iiwib3BlbmFsZXgiOiJXNDM5NjU2MzU4NCJ9LCJ0aXRsZSI6IlByZWNpc2lvbiBGYXJtaW5nOiBBIFJldmlldyBvZiBNZXRob2RzLCBUZWNobm9sb2dpZXMsIGFuZCBGdXR1cmUgUHJvc3BlY3RzIiwiYWJzdHJhY3QiOiJQcmVjaXNpb24gZmFybWluZywgb2Z0ZW4gcmVmZXJyZWQgdG8gYXMgcHJlY2lzaW9uIGFncmljdWx0dXJlLCBpcyBhIGZhcm1pbmcgbWFuYWdlbWVudCBjb25jZXB0IHRoYXQgc2Vla3MgdG8gbWF4aW1pemUgYWdyaWN1bHR1cmFsIG9wZXJhdGlvbnMgdmlhIHRoZSB1c2Ugb2YgdGVjaG5vbG9neSBpbiBvcmRlciB0byBhY2hpZXZlIGdyZWF0ZXIgbGV2ZWxzIG9mIGVmZmljaWVuY3ksIHByb2R1Y3Rpb24sIGFuZCBjb25zZXJ2YXRpb24gb2YgcmVzb3VyY2VzLiBUaGUgcHVycG9zZSBvZiB0aGlzIHN1cnZleSBzdHVkeSBpcyB0byBvZmZlciBhbiBvdmVydmlldyBvZiBwcmVjaXNpb24gZmFybWluZywgaW5jbHVkaW5nIHRvcGljcyBzdWNoIGFzIGl0cyBmdW5kYW1lbnRhbHMsIG1ldGhvZG9sb2dpZXMsIHRlY2hub2xvZ3ksIGFuZCBhcHBsaWNhdGlvbnMuIEl0IGludmVzdGlnYXRlcyB0aGUgcm9sZSB0aGF0IGEgdmFyaWV0eSBvZiB0ZWNobm9sb2dpZXMsIGluY2x1ZGluZyBnbG9iYWwgbmF2aWdhdGlvbiBzYXRlbGxpdGUgc3lzdGVtcyAoR05TUyksIGdlb2dyYXBoaWMgaW5mb3JtYXRpb24gc3lzdGVtcyAoR0lTKSwgcmVtb3RlIHNlbnNpbmcsIGFuZCBzZW5zb3IgdGVjaG5vbG9neSwgcGxheSBpbiBtYWtpbmcgaXQgcG9zc2libGUgdG8gaW1wbGVtZW50IHByZWNpc2lvbiBhZ3JpY3VsdHVyYWwgbWV0aG9kcy4gSW4gYWRkaXRpb24sIGl0IGFkZHJlc3NlcyB0aGUgYWR2YW50YWdlcywgZGlzYWR2YW50YWdlcywgYW5kIHBvdGVudGlhbCBmdXR1cmUgYXBwbGljYXRpb25zIG9mIHByZWNpc2lvbiBmYXJtaW5nIGluIHRoZSBjb250ZXh0IG9mIHNvbHZpbmcgaXNzdWVzIHJlbGF0ZWQgdG8gdGhlIHN1c3RhaW5hYmlsaXR5IG9mIGFncmljdWx0dXJlIGFuZCB0aGUgc2VjdXJpdHkgb2YgZm9vZCBzdXBwbGllcyBvbiBhIHdvcmxkd2lkZSBzY2FsZS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjIxNjEvaWplYWIuOTIuMjciLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vaWplYWIuY29tL3VwbG9hZF9kb2N1bWVudC9pc3N1ZV9maWxlcy8yN0lKRUFCLTEwNDIwMjQ4NC1QcmVjaXNpb25GYXJtaW5nLnBkZiIsImlzc3VlZERhdGUiOiIyMDI0LTAxLTAxIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiSW50ZXJuYXRpb25hbCBKb3VybmFsIG9mIEVudmlyb25tZW50IEFncmljdWx0dXJlIGFuZCBCaW90ZWNobm9sb2d5IiwiaXNzdWUiOjIsInZvbHVtZSI6OSwiaW1wYWN0RmFjdG9yIjoiMC42NSJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjI0MiwibGFzdFBhZ2UiOjI1M319LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJJbmZvZ2FpbiBQdWJsaWNhdGlvbiJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NCwiY2l0ZWRCeUNvdW50Ijo4fX0sInF1b3RlIjoiSUlJLiBCRU5FRklUUyBPRiBQUkVDSVNJT04gSVJSSUdBVElPTiAxLiBJbXByb3ZlZCBXYXRlciBVc2UgRWZmaWNpZW5jeTogUHJlY2lzaW9uIGlycmlnYXRpb24gb3B0aW1pemVzIHdhdGVyIHVzZSBlZmZpY2llbmN5IGJ5IGRlbGl2ZXJpbmcgd2F0ZXIgZGlyZWN0bHkgdG8gdGhlIHJvb3Qgem9uZSBvZiBjcm9wcywgbWluaW1pemluZyBsb3NzZXMgIGR1ZSB0byBldmFwb3JhdGlvbiwgcnVub2ZmLCBhbmQgZGVlcCBwZXJjb2xhdGlvbi4gQnkgIG1hdGNoaW5nIHdhdGVyIGFwcGxpY2F0aW9uIHJhdGVzIHRvIGNyb3Agd2F0ZXIgIHJlcXVpcmVtZW50cywgcHJlY2lzaW9uIGlycmlnYXRpb24gc3lzdGVtcyBlbnN1cmUgdGhhdCAgd2F0ZXIgaXMgdXNlZCBtb3JlIGVmZmljaWVudGx5LCBtYXhpbWl6aW5nIGNyb3AgeWllbGRzICBwZXIgdW5pdCBvZiB3YXRlciBhcHBsaWVkLiAyLiBFbmhhbmNlZCBDcm9wIFBlcmZvcm1hbmNlOiBCeSBwcm92aWRpbmcgY3JvcHMgd2l0aCBvcHRpbWFsIG1vaXN0dXJlIGxldmVscywgcHJlY2lzaW9uIGlycmlnYXRpb24gc3lzdGVtcyBwcm9tb3RlIGhlYWx0aHkgcm9vdCBkZXZlbG9wbWVudCwgYmFsYW5jZWQgZ3Jvd3RoLCAgYW5kIGltcHJvdmVkIGNyb3Agdmlnb3IuIENvbnNpc3RlbnQgc29pbCBtb2lzdHVyZSAgbGV2ZWxzIGhlbHAgcmVkdWNlIHdhdGVyIHN0cmVzcywgaW5jcmVhc2UgbnV0cmllbnQgIHVwdGFrZSwgYW5kIGVuaGFuY2UgY3JvcCByZXNpbGllbmNlIHRvIGVudmlyb25tZW50YWwgIHN0cmVzc2VzIHN1Y2ggYXMgZHJvdWdodCBhbmQgaGVhdC4gMy4gQ29zdCBTYXZpbmdzOiBQcmVjaXNpb24gaXJyaWdhdGlvbiBzeXN0ZW1zIGhlbHAgZmFybWVycyByZWR1Y2Ugd2F0ZXIsIGVuZXJneSwgYW5kIGxhYm9yIGNvc3RzIGFzc29jaWF0ZWQgd2l0aCBpcnJpZ2F0aW9uLiBCeSBvcHRpbWl6aW5nIHdhdGVyIHVzZSBlZmZpY2llbmN5IGFuZCAgbWluaW1pemluZyBpbnB1dCB3YXN0YWdlLCBwcmVjaXNpb24gaXJyaWdhdGlvbiBzeXN0ZW1zICBjYW4gbGVhZCB0byBzaWduaWZpY2FudCBjb3N0IHNhdmluZ3Mgb3ZlciB0aW1lLCAgaW1wcm92aW5nIHRoZSBwcm9maXRhYmlsaXR5IG9mIGFncmljdWx0dXJhbCBvcGVyYXRpb25zLiA0LiBFbnZpcm9ubWVudGFsIFN1c3RhaW5hYmlsaXR5OiAgUHJlY2lzaW9uICBpcnJpZ2F0aW9uIGNvbnRyaWJ1dGVzIHRvICBlbnZpcm9ubWVudGFsICBzdXN0YWluYWJpbGl0eSAgYnkgY29uc2VydmluZyB3YXRlciAgcmVzb3VyY2VzLCAgbWluaW1pemluZyAgd2F0ZXIgcG9sbHV0aW9uLCBhbmQgcmVkdWNpbmcgZ3JlZW5ob3VzZSBnYXMgZW1pc3Npb25zLiBCeSBhZG9wdGluZyBwcmVjaXNpb24gaXJyaWdhdGlvbiBwcmFjdGljZXMsIGZhcm1lcnMgY2FuICBtaW5pbWl6ZSB0aGVpciBlbnZpcm9ubWVudGFsIGZvb3RwcmludCBhbmQgY29udHJpYnV0ZSB0byB0aGUgIGxvbmctdGVybSAgc3VzdGFpbmFiaWxpdHkgIG9mICBhZ3JpY3VsdHVyYWwgcHJvZHVjdGlvbiBzeXN0ZW1zLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjQ0Ljg2NTY0MjU0NzYwNzQyLCJsZWZ0Ijo1My40MDgyNTI3MTYwNjQ0NSwid2lkdGgiOjM4LjI2MTc2ODM0MTA2NDQ1LCJoZWlnaHQiOjEuNjY5MjI3MzYxNjc5MDc3MX19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjQ2Ljg5NzkyMjUxNTg2OTE0LCJsZWZ0Ijo1Mi40MjA1NDM2NzA2NTQzLCJ3aWR0aCI6NDAuMjQwNzM3OTE1MDM5MDYsImhlaWdodCI6MS42MjU0NTQ3ODM0Mzk2MzYyfX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NDguNDkxNzkwNzcxNDg0Mzc1LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3Ljk4MDI3ODAxNTEzNjcyLCJoZWlnaHQiOjExLjAxMzcxMzgzNjY2OTkyMn19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjU5LjkzOTU0MDg2MzAzNzExLCJsZWZ0Ijo1Mi40MjA1NDM2NzA2NTQzLCJ3aWR0aCI6NDAuMjMwMjI0NjA5Mzc1LCJoZWlnaHQiOjEuNjI1NDU0NzgzNDM5NjM2Mn19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjYxLjUzMzQxNjc0ODA0Njg3NSwibGVmdCI6NTQuNjk4MzEwODUyMDUwNzgsIndpZHRoIjozNy45NjMxMjMzMjE1MzMyLCJoZWlnaHQiOjkuNDM1MTcwMTczNjQ1MDJ9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo3MS40MTY4Nzc3NDY1ODIwMywibGVmdCI6NTIuNDIwNTQzNjcwNjU0Mywid2lkdGgiOjQwLjIxMzMxMDI0MTY5OTIyLCJoZWlnaHQiOjEuNjI1NDU4MzU5NzE4MzIyOH19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjczLjAxMDc1NzQ0NjI4OTA2LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3Ljk0NTc4NTUyMjQ2MDk0LCJoZWlnaHQiOjcuODYzNzY0Mjg2MDQxMjZ9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo4MS4zMDg1NTU2MDMwMjczNCwibGVmdCI6NTIuNDIwNTQzNjcwNjU0Mywid2lkdGgiOjQwLjI0MDM3NTUxODc5ODgzLCJoZWlnaHQiOjEuNjI1NDU4MzU5NzE4MzIyOH19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjgyLjkxOTA1OTc1MzQxNzk3LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3LjkzNjQxNjYyNTk3NjU2LCJoZWlnaHQiOjEuNTc4MTMzNTgzMDY4ODQ3N319LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjg0LjQ4NjkwMDMyOTU4OTg0LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3LjkzNDc1NzIzMjY2NjAxNiwiaGVpZ2h0IjoxLjU3ODEzMzU4MzA2ODg0Nzd9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo4Ni4wNTQ3NDg1MzUxNTYyNSwibGVmdCI6NTQuNjk4MzEwODUyMDUwNzgsIndpZHRoIjozNy45Nzg0MzE3MDE2NjAxNTYsImhlaWdodCI6NC43MTM4MTkwMjY5NDcwMjE1fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6OC40MjAyOTI4NTQzMDkwODIsImxlZnQiOjEwLjAyMjE3MjkyNzg1NjQ0NSwid2lkdGgiOjM3Ljk0NjgzMDc0OTUxMTcyLCJoZWlnaHQiOjEuNTc4MTM1MjUxOTk4OTAxNH19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjkuOTg4MTMyNDc2ODA2NjQsImxlZnQiOjEwLjAyMjE3MjkyNzg1NjQ0NSwid2lkdGgiOjEzLjg2MjQ2MTA5MDA4Nzg5LCJoZWlnaHQiOjEuNTc4MTM1MjUxOTk4OTAxNH19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjI0OSJ9fV0%3D;

- Scalability: Modular for small to large farms.

### Artificial Intelligence and Machine Learning Applications

AI and machine learning analyze vast datasets from sensors and satellites to predict water needs with high accuracy, optimizing schedules for 20-40% beyond conventional precision gains and supporting sustainable intensification (Jumintono et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikp1bWludG9ubyIsImZhbWlseSI6Ikp1bWludG9ubyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJOdXJuYXdhdHkiLCJmYW1pbHkiOiJOdXJuYXdhdHkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTnVncm9obyBUcmlzbnUiLCJmYW1pbHkiOiJCcmF0YSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJEaW5pIiwiZmFtaWx5IjoiUmFobWlhdGkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSGFkaSIsImZhbWlseSI6IlN1c2lsbyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBLiBBLiIsImZhbWlseSI6Ikh1c3NlbiJ9fV0sImlkcyI6eyJpc3NuIjpbIjE0MjktNzQyNiIsIjIwODMtNDUzNSJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMjQ0MjUvandsZC4yMDI0LjE1MTc5OCIsIm9wZW5hbGV4IjoiVzQ0MDU0NzcxMDAifSwidGl0bGUiOiJDb21wYXJhdGl2ZSBhbmFseXNpcyBvZiBwcmVjaXNpb24sIGRlZmljaXQsIGFuZCBkcmlwIGlycmlnYXRpb24gZm9yIHdhdGVyIGVmZmljaWVuY3kgaW4gSXJhcSIsImFic3RyYWN0IjoiU3VzdGFpbmFibGUgaXJyaWdhdGlvbiBwcmFjdGljZXMgYXJlIGNyaXRpY2FsIGZvciBhZGRyZXNzaW5nIHRoZSBjaGFsbGVuZ2VzIG9mIHdhdGVyIHNjYXJjaXR5IGFuZCBmb29kIGluc2VjdXJpdHkgaW4gdGhlIGZhY2Ugb2YgY2xpbWF0ZSBjaGFuZ2UgYW5kIGdyb3dpbmcgZ2xvYmFsIGZvb2QgZGVtYW5kLiBUaGlzIHN0dWR5IGludmVzdGlnYXRlZCB0aGUgcG90ZW50aWFsIG9mIHByZWNpc2lvbiBpcnJpZ2F0aW9uIChQSSksIGRlZmljaXQgaXJyaWdhdGlvbiAoREkpLCBhbmQgZHJpcCBpcnJpZ2F0aW9uIChEUkkpIHRlY2huaXF1ZXMgdG8gZW5oYW5jZSBjcm9wIHByb2R1Y3Rpdml0eSBhbmQgd2F0ZXIgdXNlIGVmZmljaWVuY3kgKFdVRSkgY29tcGFyZWQgdG8gY29udmVudGlvbmFsIGZsb29kIGlycmlnYXRpb24gKENGSSkgaW4gdGhyZWUgZGlzdGluY3QgYWdyb2Vjb2xvZ2ljYWwgem9uZXMgb2YgSXJhcS4gRmllbGQgZXhwZXJpbWVudHMgd2VyZSBjb25kdWN0ZWQgdXNpbmcgYSByYW5kb21pc2VkIGNvbXBsZXRlIGJsb2NrIGRlc2lnbiB3aXRoIHdoZWF0LCBtYWl6ZSwgYW5kIHJpY2UgY3JvcHMuIFRoZSByZXN1bHRzIGRlbW9uc3RyYXRlZCB0aGF0IFBJIHNpZ25pZmljYW50bHkgaW5jcmVhc2VkIGNyb3AgZ3Jvd3RoIHBhcmFtZXRlcnMsIGdyYWluIHlpZWxkLCBhbmQgV1VFIGFjcm9zcyBhbGwgem9uZXMsIHdpdGggeWllbGQgaW1wcm92ZW1lbnRzIG9mIDMz4oCTMzglIGFuZCBXVUUgaW5jcmVhc2VzIG9mIDQ24oCTNTElIGluIGNvbnRyYXN0IHdpdGggQ0ZJLiBUaGUgREkgYW5kIERSSSB0cmVhdG1lbnRzIGFsc28gb3V0cGVyZm9ybWVkIENGSSwgYWxiZWl0IHRvIGEgbGVzc2VyIGV4dGVudC4gUmVtb3RlIHNlbnNpbmctZGVyaXZlZCB2ZWdldGF0aW9uIGluZGljZXMgc3Ryb25nbHkgY29ycmVsYXRlZCB3aXRoIGNyb3AgZ3Jvd3RoIHBhcmFtZXRlcnMgYW5kIHlpZWxkLCB3aGlsZSBoeWRyb2xvZ2ljYWwgbW9kZWxsaW5nIHJldmVhbGVkIHJlZHVjZWQgZXZhcG90cmFuc3BpcmF0aW9uIGFuZCBzdXJmYWNlIHJ1bm9mZiB1bmRlciB0aGUgUEkgdHJlYXRtZW50LiBUaGUgc3VzdGFpbmFibGUgaXJyaWdhdGlvbiBwcmFjdGljZXMgcmVzdWx0ZWQgaW4gc3Vic3RhbnRpYWwgd2F0ZXIgc2F2aW5ncyBvZiAyMOKAkzMwJSBjb21wYXJlZCB0byBDRkkuIFRoZXNlIGZpbmRpbmdzIGhpZ2hsaWdodCB0aGUgaW1wb3J0YW5jZSBvZiBhZG9wdGluZyBlZmZpY2llbnQgaXJyaWdhdGlvbiB0ZWNobmlxdWVzLCBhbG9uZyB3aXRoIGEgaG9saXN0aWMgYXBwcm9hY2ggZW5jb21wYXNzaW5nIHRlY2hub2xvZ2ljYWwgaW5ub3ZhdGlvbnMsIGNhcGFjaXR5IGJ1aWxkaW5nLCBhbmQgc3Rha2Vob2xkZXIgZW5nYWdlbWVudCwgdG8gcHJvbW90ZSB3YXRlci0gZWZmaWNpZW50IGFncmljdWx0dXJlIGFuZCBlbnN1cmUgZm9vZCBzZWN1cml0eSBpbiB3YXRlci1zY2FyY2UgcmVnaW9ucy4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjQ0MjUvandsZC4yMDI0LjE1MTc5OCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjI0NDI1L2p3bGQuMjAyNC4xNTE3OTgiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMi0xNCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkpvdXJuYWwgb2YgV2F0ZXIgYW5kIExhbmQgRGV2ZWxvcG1lbnQiLCJpbXBhY3RGYWN0b3IiOiIxLjQ2In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTMwLCJsYXN0UGFnZSI6MTM2fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkRlIEdydXl0ZXIgT3BlbiJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MzIsImNpdGVkQnlDb3VudCI6MX19LCJxdW90ZSI6IkluIHRoaXMgY29udGV4dCwgdGhlIGFkb3B0aW9uIG9mICBzdXN0YWluYWJsZSBpcnJpZ2F0aW9uIHByYWN0aWNlcyBoYXMgZW1lcmdlZCBhcyBhIGNyaXRpY2FsIHN0cmF0ZWd5ICBmb3IgZW5oYW5jaW5nIGZvb2Qgc2VjdXJpdHkgd2hpbGUgcHJvbW90aW5nIHdhdGVyIGNvbnNlcnZhdGlvbiAgaW4gYWdyaWN1bHR1cmFsIGxhbmRzY2FwZXMuICBJcnJpZ2F0aW9uIHBsYXlzIGEgdml0YWwgcm9sZSBpbiBnbG9iYWwgZm9vZCBwcm9kdWN0aW9uLCB3aXRoICBpcnJpZ2F0ZWQgYWdyaWN1bHR1cmUgYWNjb3VudGluZyBmb3IgYXBwcm94aW1hdGVseSA0MCUgb2YgdGhlICBmb29kIHN1cHBseSAoV3JhY2hpZW4gZGUsIFNjaHVsdHogYW5kIEdvbGksIDIwMjE7IFNjaG1pdHQsIFJvc2EgIGFuZCBEYWlseSwgMjAyMikuIEhvd2V2ZXIsIHRyYWRpdGlvbmFsIGlycmlnYXRpb24gbWV0aG9kcywgc3VjaCBhcyAgZmxvb2QgYW5kIGZ1cnJvdyBpcnJpZ2F0aW9uLCBhcmUgb2Z0ZW4gaW5lZmZpY2llbnQsIGxlYWRpbmcgdG8gIHN1YnN0YW50aWFsIHdhdGVyIGxvc3NlcyB0aHJvdWdoIGV2YXBvcmF0aW9uLCBydW5vZmYsIGFuZCBkZWVwICBwZXJjb2xhdGlvbiAoQW1ib21zYSwgMjAyMCkuIE1vcmVvdmVyLCB0aGUgZXhjZXNzaXZlIGFwcGxpY2EtdGlvbiBvZiB3YXRlciBjYW4gcmVzdWx0IGluIHNvaWwgc2FsaW5pc2F0aW9uLCBudXRyaWVudCBsZWFjaGluZywgYW5kICBncm91bmR3YXRlciBkZXBsZXRpb24sIGZ1cnRoZXIgY29tcHJvbWlzaW5nIHRoZSBsb25nLXRlcm0gIHN1c3RhaW5hYmlsaXR5IG9mIGFncmljdWx0dXJhbCBzeXN0ZW1zIChZaW4gZXQgYWwuLCAyMDIyKS4gVG8gYWRkcmVzcyAgdGhlc2UgY2hhbGxlbmdlcywgcmVzZWFyY2hlcnMgYW5kIHByYWN0aXRpb25lcnMgaGF2ZSBiZWVuIGV4cGxvci1pbmcgaW5ub3ZhdGl2ZSBpcnJpZ2F0aW9uIHRlY2huaXF1ZXMgYW5kIHdhdGVyIG1hbmFnZW1lbnQgIHN0cmF0ZWdpZXMgdGhhdCBjYW4gb3B0aW1pc2Ugd2F0ZXIgdXNlIGVmZmljaWVuY3kgKFdVRSkgd2hpbGUgIG1haW50YWluaW5nIG9yIGVuaGFuY2luZyBjcm9wIHByb2R1Y3Rpdml0eS4gIFByZWNpc2lvbiBpcnJpZ2F0aW9uIChQSSkgb2ZmZXJzIGEgcHJvbWlzaW5nIHNvbHV0aW9uLCAgaW52b2x2aW5nIHRoZSB0YXJnZXRlZCBhcHBsaWNhdGlvbiBvZiB3YXRlciBzcGVjaWZpY2FsbHkgdG8gdGhlICBuZWVkcyBvZiBpbmRpdmlkdWFsIHBsYW50cyBvciBjcm9wIHpvbmVzIChaaW5rZXJuYWdlbCBldCBhbC4sIDIwMjApLiAgIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyMi44ODY3MzU5MTYxMzc2OTUsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzQ2MjIxOTIzODI4MTI1LCJoZWlnaHQiOjEuNDUwNTQxMDE5NDM5Njk3M319LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI0LjM1MTEyMzgwOTgxNDQ1MywibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTI3MTYwNjQ0NSwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI1LjcyNDk4NzAzMDAyOTI5NywibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTY1MzA3NjE3MiwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI3LjA5ODg1MDI1MDI0NDE0LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjE2LjQxODE2MTM5MjIxMTkxNCwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI4LjQ3MjcwOTY1NTc2MTcyLCJsZWZ0IjoxMi4zODA4ODAzNTU4MzQ5NjEsIndpZHRoIjozNy4wMTIxODQxNDMwNjY0MDYsImhlaWdodCI6MS4zNjAwMTQ3OTYyNTcwMTl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyOS44NDY1NzI4NzU5NzY1NjIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQwNjQ4NjUxMTIzMDUsImhlaWdodCI6MS4zNjAwMTY3MDM2MDU2NTE5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzEuMjE0MDM2OTQxNTI4MzIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQ0ODU2MjYyMjA3MDMsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzIuNTg3OTAyMDY5MDkxOCwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDYwNTc4OTE4NDU3LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMzLjk2MTc2MTQ3NDYwOTM3NSwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDA0ODQ2MTkxNDA2MjUsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzUuMzM1NjI0Njk0ODI0MjIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzM5NzQ0NTY3ODcxMDk0LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjM2LjcwOTQ4NzkxNTAzOTA2LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjAwMTU5MDcyODc1OTc2NiwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC4wODMzNDczMjA1NTY2NCwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTY1MzA3NjE3MiwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOS40NTcyMTA1NDA3NzE0ODQsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQzMDQ4MDk1NzAzMTI1LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQwLjc0MDU0NzE4MDE3NTc4LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjM0NTQ1NTE2OTY3NzczNCwiaGVpZ2h0IjoxLjQ1MDU0MjgwNzU3OTA0MDV9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Mi4yMDQ5MzMxNjY1MDM5MDYsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMDAzMjUwMTIyMDcwMzEsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDMuNTc4Nzk2Mzg2NzE4NzUsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzM3OTIxMTQyNTc4MTI1LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQ0Ljg2MjEzMzAyNjEyMzA1LCJsZWZ0Ijo5LjA0NzYwMTY5OTgyOTEwMiwid2lkdGgiOjQwLjM0NTMxNDAyNTg3ODkwNiwiaGVpZ2h0IjoxLjQ1MDU0MjgwNzU3OTA0MDV9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ni4zMjY1MTkwMTI0NTExNywibGVmdCI6OS4wNDc2MDE2OTk4MjkxMDIsIndpZHRoIjoyNy45Njc0NDcyODA4ODM3OSwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ny43MDAzODIyMzI2NjYwMTYsImxlZnQiOjEyLjM4MDg3ODQ0ODQ4NjMyOCwid2lkdGgiOjM3LjAzNDc4NjIyNDM2NTIzNCwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0OS4wNzQyNDU0NTI4ODA4NiwibGVmdCI6OS4wNDc2MDE2OTk4MjkxMDIsIndpZHRoIjo0MC4zNDA2NDQ4MzY0MjU3OCwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1MC4zNTc1ODIwOTIyODUxNTYsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzQ0ODUyNDQ3NTA5NzY2LCJoZWlnaHQiOjEuNDUwNTQyODA3NTc5MDQwNX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEzMSJ9fV0%3D. These tools process variables like weather forecasts and historical yields to model ET dynamically, countering climate variability in water-stressed agriculture (Daraz et al., 2025)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlVtYXIiLCJmYW1pbHkiOiJEYXJheiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiLFoHRlZmFuIiwiZmFtaWx5IjoiQm9qbmVjIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllvdW5hcyIsImZhbWlseSI6IktoYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMDc3LTA0NzIiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzOTAvYWdyaWN1bHR1cmUxNTA1MDU1NCIsIm9wZW5hbGV4IjoiVzQ0MDgyMTk4MTEiLCJleGEiOiJodHRwczovL3d3dy5tZHBpLmNvbS8yMDc3LTA0NzIvMTUvNS81NTQifSwidGl0bGUiOiJFbmVyZ3ktRWZmaWNpZW50IFNtYXJ0IElycmlnYXRpb24gVGVjaG5vbG9naWVzOiBBIFBhdGh3YXkgdG8gV2F0ZXIgYW5kIEVuZXJneSBTdXN0YWluYWJpbGl0eSBpbiBBZ3JpY3VsdHVyZSIsImFic3RyYWN0IjoiVGhlIGFncmljdWx0dXJhbCBzZWN0b3IgZmFjZXMgY2hhbGxlbmdlcyBzdWNoIGFzIHdhdGVyIHNjYXJjaXR5LCBlbmVyZ3kgaW5lZmZpY2llbmN5LCBhbmQgZGVjbGluaW5nIHByb2R1Y3Rpdml0eSwgcGFydGljdWxhcmx5IGluIGFyaWQgcmVnaW9ucy4gVHJhZGl0aW9uYWwgaXJyaWdhdGlvbiBtZXRob2RzIGNvbnRyaWJ1dGUgdG8gcmVzb3VyY2UgZGVwbGV0aW9uIGFuZCBlbnZpcm9ubWVudGFsIGltcGFjdHMuIFNvbGFyLXBvd2VyZWQgc21hcnQgaXJyaWdhdGlvbiBzeXN0ZW1zIGludGVncmF0ZSBwcmVjaXNpb24gaXJyaWdhdGlvbiB3aXRoIHJlbmV3YWJsZSBlbmVyZ3ksIGltcHJvdmluZyB3YXRlciB1c2UgYW5kIHByb2R1Y3Rpdml0eS4gSW4gUGFraXN0YW4sIHdoZXJlIGFncmljdWx0dXJlIGNvbnRyaWJ1dGVzIDE5JSBvZiBncm9zcyBkb21lc3RpYyBwcm9kdWN0IGFuZCBlbXBsb3lzIDQwJSBvZiB0aGUgd29ya2ZvcmNlLCB0aGVzZSBjaGFsbGVuZ2VzIGFyZSBzZXZlcmUsIGVzcGVjaWFsbHkgaW4gd2F0ZXItc2NhcmNlIGFyZWFzIGxpa2UgdGhlIENob2xpc3RhbiBEZXNlcnQuIFRoaXMgc3R1ZHkgZXhhbWluZXMgdGhlIGltcGFjdCBvZiBzb2xhci1wb3dlcmVkIHNtYXJ0IGlycmlnYXRpb24gb24gYWdyaWN1bHR1cmFsIHByb2R1Y3Rpdml0eSwgd2F0ZXIgY29uc2VydmF0aW9uLCBhbmQgZW5lcmd5IGVmZmljaWVuY3kgaW4gdGhlIENob2xpc3RhbiBEZXNlcnQuIFVzaW5nIGEgcXVhbnRpdGF0aXZlIGNyb3NzLXNlY3Rpb25hbCBkZXNpZ24sIGRhdGEgd2VyZSBjb2xsZWN0ZWQgZnJvbSAzODQgZmFybWVycyB2aWEgc3RydWN0dXJlZCBxdWVzdGlvbm5haXJlcy4gU3RhdGlzdGljYWwgYW5hbHlzZXMsIGluY2x1ZGluZyBtdWx0aXBsZSBsaW5lYXIgcmVncmVzc2lvbiwgcGFpcmVkIHNhbXBsZSB0LXRlc3RzLCBhbmQgU3RydWN0dXJhbCBFcXVhdGlvbiBNb2RlbGluZyAoU0VNKSwgd2VyZSBjb25kdWN0ZWQuIEZpbmRpbmdzIHNob3cgc2lnbmlmaWNhbnQgaW1wcm92ZW1lbnRzIGluIGNyb3AgeWllbGQgKGZyb20gMy4wIHRvIDQuOCB0b25zL2hlY3RhcmUpIGFuZCByZWR1Y3Rpb25zIGluIHdhdGVyIGFuZCBlbmVyZ3kgY29uc3VtcHRpb24uIFJlZ3Jlc3Npb24gYW5hbHlzaXMgaGlnaGxpZ2h0ZWQgc3Ryb25nIHBvc2l0aXZlIGVmZmVjdHMgb24geWllbGQgYW5kIGVmZmljaWVuY3ksIHdoaWxlIFNFTSBjb25maXJtZWQgcmVkdWNlZCBlbnZpcm9ubWVudGFsIGltcGFjdCBhbmQgb3BlcmF0aW9uYWwgY29zdHMuIFRoZSBzdHVkeSBjb25jbHVkZXMgdGhhdCBzb2xhci1wb3dlcmVkIGlycmlnYXRpb24gZW5oYW5jZXMgcHJvZHVjdGl2aXR5LCBjb25zZXJ2ZXMgcmVzb3VyY2VzLCBhbmQgcHJvbW90ZXMgc3VzdGFpbmFiaWxpdHkuIFBvbGljeW1ha2VycyBzaG91bGQgcHJvdmlkZSBmaW5hbmNpYWwgaW5jZW50aXZlcywgaW52ZXN0IGluIHJlbmV3YWJsZSBpbmZyYXN0cnVjdHVyZSwgYW5kIGltcGxlbWVudCB0cmFpbmluZyBwcm9ncmFtcyB0byBzdXBwb3J0IGFkb3B0aW9uLiBDb2xsYWJvcmF0aXZlIGVmZm9ydHMgYXJlIGVzc2VudGlhbCBmb3Igc3VzdGFpbmFibGUgYWdyaWN1bHR1cmUgaW4gYXJpZCByZWdpb25zLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJpc3N1ZWREYXRlIjoiMjAyNS0wMy0wNSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkFncmljdWx0dXJlIiwiaXNzdWUiOjUsInZvbHVtZSI6MTUsImltcGFjdEZhY3RvciI6IjUuMjYifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjo1NTQsImxhc3RQYWdlIjo1NTR9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiTXVsdGlkaXNjaXBsaW5hcnkgRGlnaXRhbCBQdWJsaXNoaW5nIEluc3RpdHV0ZSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NDgsImNpdGVkQnlDb3VudCI6Mn19LCJxdW90ZSI6IjEuIEludHJvZHVjdGlvbiBUaGUgYWdyaWN1bHR1cmFsIHNlY3RvciwgYSBjb3JuZXJzdG9uZSBvZiBnbG9iYWwgZm9vZCBzZWN1cml0eSBhbmQgZWNvbm9taWMgZGV2ZWxvcG1lbnQsIGZhY2VzIG1vdW50aW5nIGNoYWxsZW5nZXMgaW4gdGhlIDIxc3QgY2VudHVyeS4gQ2xpbWF0ZSBjaGFuZ2UsIHJhcGlkIHBvcHVsYXRpb24gZ3Jvd3RoLCBhbmQgZHdpbmRsaW5nIG5hdHVyYWwgcmVzb3VyY2VzIGhhdmUgaW50ZW5zaWZpZWQgdGhlIG5lZWQgZm9yIHN1c3RhaW5hYmxlIGFncmljdWx0dXJhbCBwcmFjdGljZXMuIEFtb25nIHRoZXNlIGNoYWxsZW5nZXMsIHdhdGVyIHNjYXJjaXR5IGFuZCBlbmVyZ3kgaW5lZmZpY2llbmN5IHN0YW5kIG91dCBhcyBjcml0aWNhbCBiYXJyaWVycyB0byBwcm9kdWN0aXZpdHkgYW5kIHN1c3RhaW5hYmlsaXR5LiBHbG9iYWxseSwgYWdyaWN1bHR1cmUgYWNjb3VudHMgZm9yIGFwcHJveGltYXRlbHkgNzAlIG9mIGZyZXNod2F0ZXIgd2l0aGRyYXdhbHMgYW5kIGlzIG9uZSBvZiB0aGUgbGFyZ2VzdCBjb25zdW1lcnMgb2YgZW5lcmd5IGluIG1hbnkgcmVnaW9ucyBbIDEsIDJdLiBUcmFkaXRpb25hbCBpcnJpZ2F0aW9uIHByYWN0aWNlcywgb2Z0ZW4gaW5lZmZpY2llbnQgYW5kIHJlc291cmNlLWludGVuc2l2ZSwgZXhhY2VyYmF0ZSB0aGVzZSBpc3N1ZXMsIGxlYWRpbmcgdG8gd2F0ZXIgd2FzdGFnZSwgZW5lcmd5IG92ZXJ1c2UsIGFuZCByZWR1Y2VkIGNyb3AgeWllbGRzLiJ9XQ%3D%3D.

### Table 8: Historical Irrigation Milestones by Civilization

| Civilization | Innovation | Primary Resource |
| --- | --- | --- |
| Mesopotamia | Canal engineering | Tigris/Euphrates Rivers |
| Ancient Egypt | Land distribution/Basins | Nile River flooding |
| Indus Valley | Philosophical nature-rel. | Indus/Ganges Rivers |
| Ancient China | Engineered ditches | Slow transition pathways |
| Early Society | Surplus food production | Manual labor focus |
| Traditional | Surface gravity systems | Dominant practice for millennia |
| Medieval | Labor-intensive theory | Power concentration focus |
| Modern | Shift to precision | IoT and AI integration |

Principles of Operation

ML algorithms (e.g., random forests, neural networks) train on IoT/remote sensing data to forecast crop coefficients and stress, integrating with controllers for prescriptive irrigation (Kushwaha et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjMwLCJzdWJ0eXBlIjp7fSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTWFuaXNoIiwiZmFtaWx5IjoiS3VzaHdhaGEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2hhbmthciIsImZhbWlseSI6IlNpbmdoIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlZpamF5IEthbnQiLCJmYW1pbHkiOiJTaW5naCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTaGFzaGFuayIsImZhbWlseSI6IkR3aXZlZGkifX1dLCJpZHMiOnsiaXNzbiI6WyIyNDU2LTE4NzgiXSwiaXNibiI6W10sImRvaSI6IjEwLjIyMTYxL2lqZWFiLjkyLjI3Iiwib3BlbmFsZXgiOiJXNDM5NjU2MzU4NCJ9LCJ0aXRsZSI6IlByZWNpc2lvbiBGYXJtaW5nOiBBIFJldmlldyBvZiBNZXRob2RzLCBUZWNobm9sb2dpZXMsIGFuZCBGdXR1cmUgUHJvc3BlY3RzIiwiYWJzdHJhY3QiOiJQcmVjaXNpb24gZmFybWluZywgb2Z0ZW4gcmVmZXJyZWQgdG8gYXMgcHJlY2lzaW9uIGFncmljdWx0dXJlLCBpcyBhIGZhcm1pbmcgbWFuYWdlbWVudCBjb25jZXB0IHRoYXQgc2Vla3MgdG8gbWF4aW1pemUgYWdyaWN1bHR1cmFsIG9wZXJhdGlvbnMgdmlhIHRoZSB1c2Ugb2YgdGVjaG5vbG9neSBpbiBvcmRlciB0byBhY2hpZXZlIGdyZWF0ZXIgbGV2ZWxzIG9mIGVmZmljaWVuY3ksIHByb2R1Y3Rpb24sIGFuZCBjb25zZXJ2YXRpb24gb2YgcmVzb3VyY2VzLiBUaGUgcHVycG9zZSBvZiB0aGlzIHN1cnZleSBzdHVkeSBpcyB0byBvZmZlciBhbiBvdmVydmlldyBvZiBwcmVjaXNpb24gZmFybWluZywgaW5jbHVkaW5nIHRvcGljcyBzdWNoIGFzIGl0cyBmdW5kYW1lbnRhbHMsIG1ldGhvZG9sb2dpZXMsIHRlY2hub2xvZ3ksIGFuZCBhcHBsaWNhdGlvbnMuIEl0IGludmVzdGlnYXRlcyB0aGUgcm9sZSB0aGF0IGEgdmFyaWV0eSBvZiB0ZWNobm9sb2dpZXMsIGluY2x1ZGluZyBnbG9iYWwgbmF2aWdhdGlvbiBzYXRlbGxpdGUgc3lzdGVtcyAoR05TUyksIGdlb2dyYXBoaWMgaW5mb3JtYXRpb24gc3lzdGVtcyAoR0lTKSwgcmVtb3RlIHNlbnNpbmcsIGFuZCBzZW5zb3IgdGVjaG5vbG9neSwgcGxheSBpbiBtYWtpbmcgaXQgcG9zc2libGUgdG8gaW1wbGVtZW50IHByZWNpc2lvbiBhZ3JpY3VsdHVyYWwgbWV0aG9kcy4gSW4gYWRkaXRpb24sIGl0IGFkZHJlc3NlcyB0aGUgYWR2YW50YWdlcywgZGlzYWR2YW50YWdlcywgYW5kIHBvdGVudGlhbCBmdXR1cmUgYXBwbGljYXRpb25zIG9mIHByZWNpc2lvbiBmYXJtaW5nIGluIHRoZSBjb250ZXh0IG9mIHNvbHZpbmcgaXNzdWVzIHJlbGF0ZWQgdG8gdGhlIHN1c3RhaW5hYmlsaXR5IG9mIGFncmljdWx0dXJlIGFuZCB0aGUgc2VjdXJpdHkgb2YgZm9vZCBzdXBwbGllcyBvbiBhIHdvcmxkd2lkZSBzY2FsZS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjIxNjEvaWplYWIuOTIuMjciLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vaWplYWIuY29tL3VwbG9hZF9kb2N1bWVudC9pc3N1ZV9maWxlcy8yN0lKRUFCLTEwNDIwMjQ4NC1QcmVjaXNpb25GYXJtaW5nLnBkZiIsImlzc3VlZERhdGUiOiIyMDI0LTAxLTAxIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiSW50ZXJuYXRpb25hbCBKb3VybmFsIG9mIEVudmlyb25tZW50IEFncmljdWx0dXJlIGFuZCBCaW90ZWNobm9sb2d5IiwiaXNzdWUiOjIsInZvbHVtZSI6OSwiaW1wYWN0RmFjdG9yIjoiMC42NSJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjI0MiwibGFzdFBhZ2UiOjI1M319LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJJbmZvZ2FpbiBQdWJsaWNhdGlvbiJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NCwiY2l0ZWRCeUNvdW50Ijo4fX0sInF1b3RlIjoiSUlJLiBCRU5FRklUUyBPRiBQUkVDSVNJT04gSVJSSUdBVElPTiAxLiBJbXByb3ZlZCBXYXRlciBVc2UgRWZmaWNpZW5jeTogUHJlY2lzaW9uIGlycmlnYXRpb24gb3B0aW1pemVzIHdhdGVyIHVzZSBlZmZpY2llbmN5IGJ5IGRlbGl2ZXJpbmcgd2F0ZXIgZGlyZWN0bHkgdG8gdGhlIHJvb3Qgem9uZSBvZiBjcm9wcywgbWluaW1pemluZyBsb3NzZXMgIGR1ZSB0byBldmFwb3JhdGlvbiwgcnVub2ZmLCBhbmQgZGVlcCBwZXJjb2xhdGlvbi4gQnkgIG1hdGNoaW5nIHdhdGVyIGFwcGxpY2F0aW9uIHJhdGVzIHRvIGNyb3Agd2F0ZXIgIHJlcXVpcmVtZW50cywgcHJlY2lzaW9uIGlycmlnYXRpb24gc3lzdGVtcyBlbnN1cmUgdGhhdCAgd2F0ZXIgaXMgdXNlZCBtb3JlIGVmZmljaWVudGx5LCBtYXhpbWl6aW5nIGNyb3AgeWllbGRzICBwZXIgdW5pdCBvZiB3YXRlciBhcHBsaWVkLiAyLiBFbmhhbmNlZCBDcm9wIFBlcmZvcm1hbmNlOiBCeSBwcm92aWRpbmcgY3JvcHMgd2l0aCBvcHRpbWFsIG1vaXN0dXJlIGxldmVscywgcHJlY2lzaW9uIGlycmlnYXRpb24gc3lzdGVtcyBwcm9tb3RlIGhlYWx0aHkgcm9vdCBkZXZlbG9wbWVudCwgYmFsYW5jZWQgZ3Jvd3RoLCAgYW5kIGltcHJvdmVkIGNyb3Agdmlnb3IuIENvbnNpc3RlbnQgc29pbCBtb2lzdHVyZSAgbGV2ZWxzIGhlbHAgcmVkdWNlIHdhdGVyIHN0cmVzcywgaW5jcmVhc2UgbnV0cmllbnQgIHVwdGFrZSwgYW5kIGVuaGFuY2UgY3JvcCByZXNpbGllbmNlIHRvIGVudmlyb25tZW50YWwgIHN0cmVzc2VzIHN1Y2ggYXMgZHJvdWdodCBhbmQgaGVhdC4gMy4gQ29zdCBTYXZpbmdzOiBQcmVjaXNpb24gaXJyaWdhdGlvbiBzeXN0ZW1zIGhlbHAgZmFybWVycyByZWR1Y2Ugd2F0ZXIsIGVuZXJneSwgYW5kIGxhYm9yIGNvc3RzIGFzc29jaWF0ZWQgd2l0aCBpcnJpZ2F0aW9uLiBCeSBvcHRpbWl6aW5nIHdhdGVyIHVzZSBlZmZpY2llbmN5IGFuZCAgbWluaW1pemluZyBpbnB1dCB3YXN0YWdlLCBwcmVjaXNpb24gaXJyaWdhdGlvbiBzeXN0ZW1zICBjYW4gbGVhZCB0byBzaWduaWZpY2FudCBjb3N0IHNhdmluZ3Mgb3ZlciB0aW1lLCAgaW1wcm92aW5nIHRoZSBwcm9maXRhYmlsaXR5IG9mIGFncmljdWx0dXJhbCBvcGVyYXRpb25zLiA0LiBFbnZpcm9ubWVudGFsIFN1c3RhaW5hYmlsaXR5OiAgUHJlY2lzaW9uICBpcnJpZ2F0aW9uIGNvbnRyaWJ1dGVzIHRvICBlbnZpcm9ubWVudGFsICBzdXN0YWluYWJpbGl0eSAgYnkgY29uc2VydmluZyB3YXRlciAgcmVzb3VyY2VzLCAgbWluaW1pemluZyAgd2F0ZXIgcG9sbHV0aW9uLCBhbmQgcmVkdWNpbmcgZ3JlZW5ob3VzZSBnYXMgZW1pc3Npb25zLiBCeSBhZG9wdGluZyBwcmVjaXNpb24gaXJyaWdhdGlvbiBwcmFjdGljZXMsIGZhcm1lcnMgY2FuICBtaW5pbWl6ZSB0aGVpciBlbnZpcm9ubWVudGFsIGZvb3RwcmludCBhbmQgY29udHJpYnV0ZSB0byB0aGUgIGxvbmctdGVybSAgc3VzdGFpbmFiaWxpdHkgIG9mICBhZ3JpY3VsdHVyYWwgcHJvZHVjdGlvbiBzeXN0ZW1zLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjQ0Ljg2NTY0MjU0NzYwNzQyLCJsZWZ0Ijo1My40MDgyNTI3MTYwNjQ0NSwid2lkdGgiOjM4LjI2MTc2ODM0MTA2NDQ1LCJoZWlnaHQiOjEuNjY5MjI3MzYxNjc5MDc3MX19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjQ2Ljg5NzkyMjUxNTg2OTE0LCJsZWZ0Ijo1Mi40MjA1NDM2NzA2NTQzLCJ3aWR0aCI6NDAuMjQwNzM3OTE1MDM5MDYsImhlaWdodCI6MS42MjU0NTQ3ODM0Mzk2MzYyfX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NDguNDkxNzkwNzcxNDg0Mzc1LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3Ljk4MDI3ODAxNTEzNjcyLCJoZWlnaHQiOjExLjAxMzcxMzgzNjY2OTkyMn19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjU5LjkzOTU0MDg2MzAzNzExLCJsZWZ0Ijo1Mi40MjA1NDM2NzA2NTQzLCJ3aWR0aCI6NDAuMjMwMjI0NjA5Mzc1LCJoZWlnaHQiOjEuNjI1NDU0NzgzNDM5NjM2Mn19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjYxLjUzMzQxNjc0ODA0Njg3NSwibGVmdCI6NTQuNjk4MzEwODUyMDUwNzgsIndpZHRoIjozNy45NjMxMjMzMjE1MzMyLCJoZWlnaHQiOjkuNDM1MTcwMTczNjQ1MDJ9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo3MS40MTY4Nzc3NDY1ODIwMywibGVmdCI6NTIuNDIwNTQzNjcwNjU0Mywid2lkdGgiOjQwLjIxMzMxMDI0MTY5OTIyLCJoZWlnaHQiOjEuNjI1NDU4MzU5NzE4MzIyOH19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjczLjAxMDc1NzQ0NjI4OTA2LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3Ljk0NTc4NTUyMjQ2MDk0LCJoZWlnaHQiOjcuODYzNzY0Mjg2MDQxMjZ9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo4MS4zMDg1NTU2MDMwMjczNCwibGVmdCI6NTIuNDIwNTQzNjcwNjU0Mywid2lkdGgiOjQwLjI0MDM3NTUxODc5ODgzLCJoZWlnaHQiOjEuNjI1NDU4MzU5NzE4MzIyOH19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjgyLjkxOTA1OTc1MzQxNzk3LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3LjkzNjQxNjYyNTk3NjU2LCJoZWlnaHQiOjEuNTc4MTMzNTgzMDY4ODQ3N319LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjg0LjQ4NjkwMDMyOTU4OTg0LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3LjkzNDc1NzIzMjY2NjAxNiwiaGVpZ2h0IjoxLjU3ODEzMzU4MzA2ODg0Nzd9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo4Ni4wNTQ3NDg1MzUxNTYyNSwibGVmdCI6NTQuNjk4MzEwODUyMDUwNzgsIndpZHRoIjozNy45Nzg0MzE3MDE2NjAxNTYsImhlaWdodCI6NC43MTM4MTkwMjY5NDcwMjE1fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6OC40MjAyOTI4NTQzMDkwODIsImxlZnQiOjEwLjAyMjE3MjkyNzg1NjQ0NSwid2lkdGgiOjM3Ljk0NjgzMDc0OTUxMTcyLCJoZWlnaHQiOjEuNTc4MTM1MjUxOTk4OTAxNH19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjkuOTg4MTMyNDc2ODA2NjQsImxlZnQiOjEwLjAyMjE3MjkyNzg1NjQ0NSwid2lkdGgiOjEzLjg2MjQ2MTA5MDA4Nzg5LCJoZWlnaHQiOjEuNTc4MTM1MjUxOTk4OTAxNH19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjI0OSJ9fV0%3D.

Advantages and Efficiency Gains

- Forecasting prowess: Anticipates demands, slashing overuse (Kushwaha et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjMwLCJzdWJ0eXBlIjp7fSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTWFuaXNoIiwiZmFtaWx5IjoiS3VzaHdhaGEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2hhbmthciIsImZhbWlseSI6IlNpbmdoIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlZpamF5IEthbnQiLCJmYW1pbHkiOiJTaW5naCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTaGFzaGFuayIsImZhbWlseSI6IkR3aXZlZGkifX1dLCJpZHMiOnsiaXNzbiI6WyIyNDU2LTE4NzgiXSwiaXNibiI6W10sImRvaSI6IjEwLjIyMTYxL2lqZWFiLjkyLjI3Iiwib3BlbmFsZXgiOiJXNDM5NjU2MzU4NCJ9LCJ0aXRsZSI6IlByZWNpc2lvbiBGYXJtaW5nOiBBIFJldmlldyBvZiBNZXRob2RzLCBUZWNobm9sb2dpZXMsIGFuZCBGdXR1cmUgUHJvc3BlY3RzIiwiYWJzdHJhY3QiOiJQcmVjaXNpb24gZmFybWluZywgb2Z0ZW4gcmVmZXJyZWQgdG8gYXMgcHJlY2lzaW9uIGFncmljdWx0dXJlLCBpcyBhIGZhcm1pbmcgbWFuYWdlbWVudCBjb25jZXB0IHRoYXQgc2Vla3MgdG8gbWF4aW1pemUgYWdyaWN1bHR1cmFsIG9wZXJhdGlvbnMgdmlhIHRoZSB1c2Ugb2YgdGVjaG5vbG9neSBpbiBvcmRlciB0byBhY2hpZXZlIGdyZWF0ZXIgbGV2ZWxzIG9mIGVmZmljaWVuY3ksIHByb2R1Y3Rpb24sIGFuZCBjb25zZXJ2YXRpb24gb2YgcmVzb3VyY2VzLiBUaGUgcHVycG9zZSBvZiB0aGlzIHN1cnZleSBzdHVkeSBpcyB0byBvZmZlciBhbiBvdmVydmlldyBvZiBwcmVjaXNpb24gZmFybWluZywgaW5jbHVkaW5nIHRvcGljcyBzdWNoIGFzIGl0cyBmdW5kYW1lbnRhbHMsIG1ldGhvZG9sb2dpZXMsIHRlY2hub2xvZ3ksIGFuZCBhcHBsaWNhdGlvbnMuIEl0IGludmVzdGlnYXRlcyB0aGUgcm9sZSB0aGF0IGEgdmFyaWV0eSBvZiB0ZWNobm9sb2dpZXMsIGluY2x1ZGluZyBnbG9iYWwgbmF2aWdhdGlvbiBzYXRlbGxpdGUgc3lzdGVtcyAoR05TUyksIGdlb2dyYXBoaWMgaW5mb3JtYXRpb24gc3lzdGVtcyAoR0lTKSwgcmVtb3RlIHNlbnNpbmcsIGFuZCBzZW5zb3IgdGVjaG5vbG9neSwgcGxheSBpbiBtYWtpbmcgaXQgcG9zc2libGUgdG8gaW1wbGVtZW50IHByZWNpc2lvbiBhZ3JpY3VsdHVyYWwgbWV0aG9kcy4gSW4gYWRkaXRpb24sIGl0IGFkZHJlc3NlcyB0aGUgYWR2YW50YWdlcywgZGlzYWR2YW50YWdlcywgYW5kIHBvdGVudGlhbCBmdXR1cmUgYXBwbGljYXRpb25zIG9mIHByZWNpc2lvbiBmYXJtaW5nIGluIHRoZSBjb250ZXh0IG9mIHNvbHZpbmcgaXNzdWVzIHJlbGF0ZWQgdG8gdGhlIHN1c3RhaW5hYmlsaXR5IG9mIGFncmljdWx0dXJlIGFuZCB0aGUgc2VjdXJpdHkgb2YgZm9vZCBzdXBwbGllcyBvbiBhIHdvcmxkd2lkZSBzY2FsZS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjIxNjEvaWplYWIuOTIuMjciLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vaWplYWIuY29tL3VwbG9hZF9kb2N1bWVudC9pc3N1ZV9maWxlcy8yN0lKRUFCLTEwNDIwMjQ4NC1QcmVjaXNpb25GYXJtaW5nLnBkZiIsImlzc3VlZERhdGUiOiIyMDI0LTAxLTAxIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiSW50ZXJuYXRpb25hbCBKb3VybmFsIG9mIEVudmlyb25tZW50IEFncmljdWx0dXJlIGFuZCBCaW90ZWNobm9sb2d5IiwiaXNzdWUiOjIsInZvbHVtZSI6OSwiaW1wYWN0RmFjdG9yIjoiMC42NSJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjI0MiwibGFzdFBhZ2UiOjI1M319LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJJbmZvZ2FpbiBQdWJsaWNhdGlvbiJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NCwiY2l0ZWRCeUNvdW50Ijo4fX0sInF1b3RlIjoiSUlJLiBCRU5FRklUUyBPRiBQUkVDSVNJT04gSVJSSUdBVElPTiAxLiBJbXByb3ZlZCBXYXRlciBVc2UgRWZmaWNpZW5jeTogUHJlY2lzaW9uIGlycmlnYXRpb24gb3B0aW1pemVzIHdhdGVyIHVzZSBlZmZpY2llbmN5IGJ5IGRlbGl2ZXJpbmcgd2F0ZXIgZGlyZWN0bHkgdG8gdGhlIHJvb3Qgem9uZSBvZiBjcm9wcywgbWluaW1pemluZyBsb3NzZXMgIGR1ZSB0byBldmFwb3JhdGlvbiwgcnVub2ZmLCBhbmQgZGVlcCBwZXJjb2xhdGlvbi4gQnkgIG1hdGNoaW5nIHdhdGVyIGFwcGxpY2F0aW9uIHJhdGVzIHRvIGNyb3Agd2F0ZXIgIHJlcXVpcmVtZW50cywgcHJlY2lzaW9uIGlycmlnYXRpb24gc3lzdGVtcyBlbnN1cmUgdGhhdCAgd2F0ZXIgaXMgdXNlZCBtb3JlIGVmZmljaWVudGx5LCBtYXhpbWl6aW5nIGNyb3AgeWllbGRzICBwZXIgdW5pdCBvZiB3YXRlciBhcHBsaWVkLiAyLiBFbmhhbmNlZCBDcm9wIFBlcmZvcm1hbmNlOiBCeSBwcm92aWRpbmcgY3JvcHMgd2l0aCBvcHRpbWFsIG1vaXN0dXJlIGxldmVscywgcHJlY2lzaW9uIGlycmlnYXRpb24gc3lzdGVtcyBwcm9tb3RlIGhlYWx0aHkgcm9vdCBkZXZlbG9wbWVudCwgYmFsYW5jZWQgZ3Jvd3RoLCAgYW5kIGltcHJvdmVkIGNyb3Agdmlnb3IuIENvbnNpc3RlbnQgc29pbCBtb2lzdHVyZSAgbGV2ZWxzIGhlbHAgcmVkdWNlIHdhdGVyIHN0cmVzcywgaW5jcmVhc2UgbnV0cmllbnQgIHVwdGFrZSwgYW5kIGVuaGFuY2UgY3JvcCByZXNpbGllbmNlIHRvIGVudmlyb25tZW50YWwgIHN0cmVzc2VzIHN1Y2ggYXMgZHJvdWdodCBhbmQgaGVhdC4gMy4gQ29zdCBTYXZpbmdzOiBQcmVjaXNpb24gaXJyaWdhdGlvbiBzeXN0ZW1zIGhlbHAgZmFybWVycyByZWR1Y2Ugd2F0ZXIsIGVuZXJneSwgYW5kIGxhYm9yIGNvc3RzIGFzc29jaWF0ZWQgd2l0aCBpcnJpZ2F0aW9uLiBCeSBvcHRpbWl6aW5nIHdhdGVyIHVzZSBlZmZpY2llbmN5IGFuZCAgbWluaW1pemluZyBpbnB1dCB3YXN0YWdlLCBwcmVjaXNpb24gaXJyaWdhdGlvbiBzeXN0ZW1zICBjYW4gbGVhZCB0byBzaWduaWZpY2FudCBjb3N0IHNhdmluZ3Mgb3ZlciB0aW1lLCAgaW1wcm92aW5nIHRoZSBwcm9maXRhYmlsaXR5IG9mIGFncmljdWx0dXJhbCBvcGVyYXRpb25zLiA0LiBFbnZpcm9ubWVudGFsIFN1c3RhaW5hYmlsaXR5OiAgUHJlY2lzaW9uICBpcnJpZ2F0aW9uIGNvbnRyaWJ1dGVzIHRvICBlbnZpcm9ubWVudGFsICBzdXN0YWluYWJpbGl0eSAgYnkgY29uc2VydmluZyB3YXRlciAgcmVzb3VyY2VzLCAgbWluaW1pemluZyAgd2F0ZXIgcG9sbHV0aW9uLCBhbmQgcmVkdWNpbmcgZ3JlZW5ob3VzZSBnYXMgZW1pc3Npb25zLiBCeSBhZG9wdGluZyBwcmVjaXNpb24gaXJyaWdhdGlvbiBwcmFjdGljZXMsIGZhcm1lcnMgY2FuICBtaW5pbWl6ZSB0aGVpciBlbnZpcm9ubWVudGFsIGZvb3RwcmludCBhbmQgY29udHJpYnV0ZSB0byB0aGUgIGxvbmctdGVybSAgc3VzdGFpbmFiaWxpdHkgIG9mICBhZ3JpY3VsdHVyYWwgcHJvZHVjdGlvbiBzeXN0ZW1zLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjQ0Ljg2NTY0MjU0NzYwNzQyLCJsZWZ0Ijo1My40MDgyNTI3MTYwNjQ0NSwid2lkdGgiOjM4LjI2MTc2ODM0MTA2NDQ1LCJoZWlnaHQiOjEuNjY5MjI3MzYxNjc5MDc3MX19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjQ2Ljg5NzkyMjUxNTg2OTE0LCJsZWZ0Ijo1Mi40MjA1NDM2NzA2NTQzLCJ3aWR0aCI6NDAuMjQwNzM3OTE1MDM5MDYsImhlaWdodCI6MS42MjU0NTQ3ODM0Mzk2MzYyfX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NDguNDkxNzkwNzcxNDg0Mzc1LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3Ljk4MDI3ODAxNTEzNjcyLCJoZWlnaHQiOjExLjAxMzcxMzgzNjY2OTkyMn19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjU5LjkzOTU0MDg2MzAzNzExLCJsZWZ0Ijo1Mi40MjA1NDM2NzA2NTQzLCJ3aWR0aCI6NDAuMjMwMjI0NjA5Mzc1LCJoZWlnaHQiOjEuNjI1NDU0NzgzNDM5NjM2Mn19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjYxLjUzMzQxNjc0ODA0Njg3NSwibGVmdCI6NTQuNjk4MzEwODUyMDUwNzgsIndpZHRoIjozNy45NjMxMjMzMjE1MzMyLCJoZWlnaHQiOjkuNDM1MTcwMTczNjQ1MDJ9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo3MS40MTY4Nzc3NDY1ODIwMywibGVmdCI6NTIuNDIwNTQzNjcwNjU0Mywid2lkdGgiOjQwLjIxMzMxMDI0MTY5OTIyLCJoZWlnaHQiOjEuNjI1NDU4MzU5NzE4MzIyOH19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjczLjAxMDc1NzQ0NjI4OTA2LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3Ljk0NTc4NTUyMjQ2MDk0LCJoZWlnaHQiOjcuODYzNzY0Mjg2MDQxMjZ9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo4MS4zMDg1NTU2MDMwMjczNCwibGVmdCI6NTIuNDIwNTQzNjcwNjU0Mywid2lkdGgiOjQwLjI0MDM3NTUxODc5ODgzLCJoZWlnaHQiOjEuNjI1NDU4MzU5NzE4MzIyOH19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjgyLjkxOTA1OTc1MzQxNzk3LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3LjkzNjQxNjYyNTk3NjU2LCJoZWlnaHQiOjEuNTc4MTMzNTgzMDY4ODQ3N319LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjg0LjQ4NjkwMDMyOTU4OTg0LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3LjkzNDc1NzIzMjY2NjAxNiwiaGVpZ2h0IjoxLjU3ODEzMzU4MzA2ODg0Nzd9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo4Ni4wNTQ3NDg1MzUxNTYyNSwibGVmdCI6NTQuNjk4MzEwODUyMDUwNzgsIndpZHRoIjozNy45Nzg0MzE3MDE2NjAxNTYsImhlaWdodCI6NC43MTM4MTkwMjY5NDcwMjE1fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6OC40MjAyOTI4NTQzMDkwODIsImxlZnQiOjEwLjAyMjE3MjkyNzg1NjQ0NSwid2lkdGgiOjM3Ljk0NjgzMDc0OTUxMTcyLCJoZWlnaHQiOjEuNTc4MTM1MjUxOTk4OTAxNH19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjkuOTg4MTMyNDc2ODA2NjQsImxlZnQiOjEwLjAyMjE3MjkyNzg1NjQ0NSwid2lkdGgiOjEzLjg2MjQ2MTA5MDA4Nzg5LCJoZWlnaHQiOjEuNTc4MTM1MjUxOTk4OTAxNH19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjI0OSJ9fV0%3D;

- Adaptive learning: Improves over time, enhancing yields (Jumintono et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikp1bWludG9ubyIsImZhbWlseSI6Ikp1bWludG9ubyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJOdXJuYXdhdHkiLCJmYW1pbHkiOiJOdXJuYXdhdHkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTnVncm9obyBUcmlzbnUiLCJmYW1pbHkiOiJCcmF0YSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJEaW5pIiwiZmFtaWx5IjoiUmFobWlhdGkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSGFkaSIsImZhbWlseSI6IlN1c2lsbyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBLiBBLiIsImZhbWlseSI6Ikh1c3NlbiJ9fV0sImlkcyI6eyJpc3NuIjpbIjE0MjktNzQyNiIsIjIwODMtNDUzNSJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMjQ0MjUvandsZC4yMDI0LjE1MTc5OCIsIm9wZW5hbGV4IjoiVzQ0MDU0NzcxMDAifSwidGl0bGUiOiJDb21wYXJhdGl2ZSBhbmFseXNpcyBvZiBwcmVjaXNpb24sIGRlZmljaXQsIGFuZCBkcmlwIGlycmlnYXRpb24gZm9yIHdhdGVyIGVmZmljaWVuY3kgaW4gSXJhcSIsImFic3RyYWN0IjoiU3VzdGFpbmFibGUgaXJyaWdhdGlvbiBwcmFjdGljZXMgYXJlIGNyaXRpY2FsIGZvciBhZGRyZXNzaW5nIHRoZSBjaGFsbGVuZ2VzIG9mIHdhdGVyIHNjYXJjaXR5IGFuZCBmb29kIGluc2VjdXJpdHkgaW4gdGhlIGZhY2Ugb2YgY2xpbWF0ZSBjaGFuZ2UgYW5kIGdyb3dpbmcgZ2xvYmFsIGZvb2QgZGVtYW5kLiBUaGlzIHN0dWR5IGludmVzdGlnYXRlZCB0aGUgcG90ZW50aWFsIG9mIHByZWNpc2lvbiBpcnJpZ2F0aW9uIChQSSksIGRlZmljaXQgaXJyaWdhdGlvbiAoREkpLCBhbmQgZHJpcCBpcnJpZ2F0aW9uIChEUkkpIHRlY2huaXF1ZXMgdG8gZW5oYW5jZSBjcm9wIHByb2R1Y3Rpdml0eSBhbmQgd2F0ZXIgdXNlIGVmZmljaWVuY3kgKFdVRSkgY29tcGFyZWQgdG8gY29udmVudGlvbmFsIGZsb29kIGlycmlnYXRpb24gKENGSSkgaW4gdGhyZWUgZGlzdGluY3QgYWdyb2Vjb2xvZ2ljYWwgem9uZXMgb2YgSXJhcS4gRmllbGQgZXhwZXJpbWVudHMgd2VyZSBjb25kdWN0ZWQgdXNpbmcgYSByYW5kb21pc2VkIGNvbXBsZXRlIGJsb2NrIGRlc2lnbiB3aXRoIHdoZWF0LCBtYWl6ZSwgYW5kIHJpY2UgY3JvcHMuIFRoZSByZXN1bHRzIGRlbW9uc3RyYXRlZCB0aGF0IFBJIHNpZ25pZmljYW50bHkgaW5jcmVhc2VkIGNyb3AgZ3Jvd3RoIHBhcmFtZXRlcnMsIGdyYWluIHlpZWxkLCBhbmQgV1VFIGFjcm9zcyBhbGwgem9uZXMsIHdpdGggeWllbGQgaW1wcm92ZW1lbnRzIG9mIDMz4oCTMzglIGFuZCBXVUUgaW5jcmVhc2VzIG9mIDQ24oCTNTElIGluIGNvbnRyYXN0IHdpdGggQ0ZJLiBUaGUgREkgYW5kIERSSSB0cmVhdG1lbnRzIGFsc28gb3V0cGVyZm9ybWVkIENGSSwgYWxiZWl0IHRvIGEgbGVzc2VyIGV4dGVudC4gUmVtb3RlIHNlbnNpbmctZGVyaXZlZCB2ZWdldGF0aW9uIGluZGljZXMgc3Ryb25nbHkgY29ycmVsYXRlZCB3aXRoIGNyb3AgZ3Jvd3RoIHBhcmFtZXRlcnMgYW5kIHlpZWxkLCB3aGlsZSBoeWRyb2xvZ2ljYWwgbW9kZWxsaW5nIHJldmVhbGVkIHJlZHVjZWQgZXZhcG90cmFuc3BpcmF0aW9uIGFuZCBzdXJmYWNlIHJ1bm9mZiB1bmRlciB0aGUgUEkgdHJlYXRtZW50LiBUaGUgc3VzdGFpbmFibGUgaXJyaWdhdGlvbiBwcmFjdGljZXMgcmVzdWx0ZWQgaW4gc3Vic3RhbnRpYWwgd2F0ZXIgc2F2aW5ncyBvZiAyMOKAkzMwJSBjb21wYXJlZCB0byBDRkkuIFRoZXNlIGZpbmRpbmdzIGhpZ2hsaWdodCB0aGUgaW1wb3J0YW5jZSBvZiBhZG9wdGluZyBlZmZpY2llbnQgaXJyaWdhdGlvbiB0ZWNobmlxdWVzLCBhbG9uZyB3aXRoIGEgaG9saXN0aWMgYXBwcm9hY2ggZW5jb21wYXNzaW5nIHRlY2hub2xvZ2ljYWwgaW5ub3ZhdGlvbnMsIGNhcGFjaXR5IGJ1aWxkaW5nLCBhbmQgc3Rha2Vob2xkZXIgZW5nYWdlbWVudCwgdG8gcHJvbW90ZSB3YXRlci0gZWZmaWNpZW50IGFncmljdWx0dXJlIGFuZCBlbnN1cmUgZm9vZCBzZWN1cml0eSBpbiB3YXRlci1zY2FyY2UgcmVnaW9ucy4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjQ0MjUvandsZC4yMDI0LjE1MTc5OCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjI0NDI1L2p3bGQuMjAyNC4xNTE3OTgiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMi0xNCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkpvdXJuYWwgb2YgV2F0ZXIgYW5kIExhbmQgRGV2ZWxvcG1lbnQiLCJpbXBhY3RGYWN0b3IiOiIxLjQ2In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTMwLCJsYXN0UGFnZSI6MTM2fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkRlIEdydXl0ZXIgT3BlbiJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MzIsImNpdGVkQnlDb3VudCI6MX19LCJxdW90ZSI6IkluIHRoaXMgY29udGV4dCwgdGhlIGFkb3B0aW9uIG9mICBzdXN0YWluYWJsZSBpcnJpZ2F0aW9uIHByYWN0aWNlcyBoYXMgZW1lcmdlZCBhcyBhIGNyaXRpY2FsIHN0cmF0ZWd5ICBmb3IgZW5oYW5jaW5nIGZvb2Qgc2VjdXJpdHkgd2hpbGUgcHJvbW90aW5nIHdhdGVyIGNvbnNlcnZhdGlvbiAgaW4gYWdyaWN1bHR1cmFsIGxhbmRzY2FwZXMuICBJcnJpZ2F0aW9uIHBsYXlzIGEgdml0YWwgcm9sZSBpbiBnbG9iYWwgZm9vZCBwcm9kdWN0aW9uLCB3aXRoICBpcnJpZ2F0ZWQgYWdyaWN1bHR1cmUgYWNjb3VudGluZyBmb3IgYXBwcm94aW1hdGVseSA0MCUgb2YgdGhlICBmb29kIHN1cHBseSAoV3JhY2hpZW4gZGUsIFNjaHVsdHogYW5kIEdvbGksIDIwMjE7IFNjaG1pdHQsIFJvc2EgIGFuZCBEYWlseSwgMjAyMikuIEhvd2V2ZXIsIHRyYWRpdGlvbmFsIGlycmlnYXRpb24gbWV0aG9kcywgc3VjaCBhcyAgZmxvb2QgYW5kIGZ1cnJvdyBpcnJpZ2F0aW9uLCBhcmUgb2Z0ZW4gaW5lZmZpY2llbnQsIGxlYWRpbmcgdG8gIHN1YnN0YW50aWFsIHdhdGVyIGxvc3NlcyB0aHJvdWdoIGV2YXBvcmF0aW9uLCBydW5vZmYsIGFuZCBkZWVwICBwZXJjb2xhdGlvbiAoQW1ib21zYSwgMjAyMCkuIE1vcmVvdmVyLCB0aGUgZXhjZXNzaXZlIGFwcGxpY2EtdGlvbiBvZiB3YXRlciBjYW4gcmVzdWx0IGluIHNvaWwgc2FsaW5pc2F0aW9uLCBudXRyaWVudCBsZWFjaGluZywgYW5kICBncm91bmR3YXRlciBkZXBsZXRpb24sIGZ1cnRoZXIgY29tcHJvbWlzaW5nIHRoZSBsb25nLXRlcm0gIHN1c3RhaW5hYmlsaXR5IG9mIGFncmljdWx0dXJhbCBzeXN0ZW1zIChZaW4gZXQgYWwuLCAyMDIyKS4gVG8gYWRkcmVzcyAgdGhlc2UgY2hhbGxlbmdlcywgcmVzZWFyY2hlcnMgYW5kIHByYWN0aXRpb25lcnMgaGF2ZSBiZWVuIGV4cGxvci1pbmcgaW5ub3ZhdGl2ZSBpcnJpZ2F0aW9uIHRlY2huaXF1ZXMgYW5kIHdhdGVyIG1hbmFnZW1lbnQgIHN0cmF0ZWdpZXMgdGhhdCBjYW4gb3B0aW1pc2Ugd2F0ZXIgdXNlIGVmZmljaWVuY3kgKFdVRSkgd2hpbGUgIG1haW50YWluaW5nIG9yIGVuaGFuY2luZyBjcm9wIHByb2R1Y3Rpdml0eS4gIFByZWNpc2lvbiBpcnJpZ2F0aW9uIChQSSkgb2ZmZXJzIGEgcHJvbWlzaW5nIHNvbHV0aW9uLCAgaW52b2x2aW5nIHRoZSB0YXJnZXRlZCBhcHBsaWNhdGlvbiBvZiB3YXRlciBzcGVjaWZpY2FsbHkgdG8gdGhlICBuZWVkcyBvZiBpbmRpdmlkdWFsIHBsYW50cyBvciBjcm9wIHpvbmVzIChaaW5rZXJuYWdlbCBldCBhbC4sIDIwMjApLiAgIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyMi44ODY3MzU5MTYxMzc2OTUsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzQ2MjIxOTIzODI4MTI1LCJoZWlnaHQiOjEuNDUwNTQxMDE5NDM5Njk3M319LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI0LjM1MTEyMzgwOTgxNDQ1MywibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTI3MTYwNjQ0NSwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI1LjcyNDk4NzAzMDAyOTI5NywibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTY1MzA3NjE3MiwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI3LjA5ODg1MDI1MDI0NDE0LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjE2LjQxODE2MTM5MjIxMTkxNCwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI4LjQ3MjcwOTY1NTc2MTcyLCJsZWZ0IjoxMi4zODA4ODAzNTU4MzQ5NjEsIndpZHRoIjozNy4wMTIxODQxNDMwNjY0MDYsImhlaWdodCI6MS4zNjAwMTQ3OTYyNTcwMTl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyOS44NDY1NzI4NzU5NzY1NjIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQwNjQ4NjUxMTIzMDUsImhlaWdodCI6MS4zNjAwMTY3MDM2MDU2NTE5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzEuMjE0MDM2OTQxNTI4MzIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQ0ODU2MjYyMjA3MDMsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzIuNTg3OTAyMDY5MDkxOCwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDYwNTc4OTE4NDU3LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMzLjk2MTc2MTQ3NDYwOTM3NSwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDA0ODQ2MTkxNDA2MjUsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzUuMzM1NjI0Njk0ODI0MjIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzM5NzQ0NTY3ODcxMDk0LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjM2LjcwOTQ4NzkxNTAzOTA2LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjAwMTU5MDcyODc1OTc2NiwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC4wODMzNDczMjA1NTY2NCwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTY1MzA3NjE3MiwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOS40NTcyMTA1NDA3NzE0ODQsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQzMDQ4MDk1NzAzMTI1LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQwLjc0MDU0NzE4MDE3NTc4LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjM0NTQ1NTE2OTY3NzczNCwiaGVpZ2h0IjoxLjQ1MDU0MjgwNzU3OTA0MDV9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Mi4yMDQ5MzMxNjY1MDM5MDYsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMDAzMjUwMTIyMDcwMzEsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDMuNTc4Nzk2Mzg2NzE4NzUsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzM3OTIxMTQyNTc4MTI1LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQ0Ljg2MjEzMzAyNjEyMzA1LCJsZWZ0Ijo5LjA0NzYwMTY5OTgyOTEwMiwid2lkdGgiOjQwLjM0NTMxNDAyNTg3ODkwNiwiaGVpZ2h0IjoxLjQ1MDU0MjgwNzU3OTA0MDV9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ni4zMjY1MTkwMTI0NTExNywibGVmdCI6OS4wNDc2MDE2OTk4MjkxMDIsIndpZHRoIjoyNy45Njc0NDcyODA4ODM3OSwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ny43MDAzODIyMzI2NjYwMTYsImxlZnQiOjEyLjM4MDg3ODQ0ODQ4NjMyOCwid2lkdGgiOjM3LjAzNDc4NjIyNDM2NTIzNCwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0OS4wNzQyNDU0NTI4ODA4NiwibGVmdCI6OS4wNDc2MDE2OTk4MjkxMDIsIndpZHRoIjo0MC4zNDA2NDQ4MzY0MjU3OCwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1MC4zNTc1ODIwOTIyODUxNTYsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzQ0ODUyNDQ3NTA5NzY2LCJoZWlnaHQiOjEuNDUwNTQyODA3NTc5MDQwNX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEzMSJ9fV0%3D;

- Holistic optimization: Incorporates energy and nutrient synergies.

## Water Management Strategies for Sustainable Agriculture

### Water Harvesting and Storage

Water harvesting captures runoff via structures like check dams and farm ponds, storing it for supplemental irrigation to extend growing seasons and boost resilience, achieving up to 50% yield increases in rainfed systems while conserving groundwater (Daraz et al., 2025)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlVtYXIiLCJmYW1pbHkiOiJEYXJheiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiLFoHRlZmFuIiwiZmFtaWx5IjoiQm9qbmVjIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllvdW5hcyIsImZhbWlseSI6IktoYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMDc3LTA0NzIiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzOTAvYWdyaWN1bHR1cmUxNTA1MDU1NCIsIm9wZW5hbGV4IjoiVzQ0MDgyMTk4MTEiLCJleGEiOiJodHRwczovL3d3dy5tZHBpLmNvbS8yMDc3LTA0NzIvMTUvNS81NTQifSwidGl0bGUiOiJFbmVyZ3ktRWZmaWNpZW50IFNtYXJ0IElycmlnYXRpb24gVGVjaG5vbG9naWVzOiBBIFBhdGh3YXkgdG8gV2F0ZXIgYW5kIEVuZXJneSBTdXN0YWluYWJpbGl0eSBpbiBBZ3JpY3VsdHVyZSIsImFic3RyYWN0IjoiVGhlIGFncmljdWx0dXJhbCBzZWN0b3IgZmFjZXMgY2hhbGxlbmdlcyBzdWNoIGFzIHdhdGVyIHNjYXJjaXR5LCBlbmVyZ3kgaW5lZmZpY2llbmN5LCBhbmQgZGVjbGluaW5nIHByb2R1Y3Rpdml0eSwgcGFydGljdWxhcmx5IGluIGFyaWQgcmVnaW9ucy4gVHJhZGl0aW9uYWwgaXJyaWdhdGlvbiBtZXRob2RzIGNvbnRyaWJ1dGUgdG8gcmVzb3VyY2UgZGVwbGV0aW9uIGFuZCBlbnZpcm9ubWVudGFsIGltcGFjdHMuIFNvbGFyLXBvd2VyZWQgc21hcnQgaXJyaWdhdGlvbiBzeXN0ZW1zIGludGVncmF0ZSBwcmVjaXNpb24gaXJyaWdhdGlvbiB3aXRoIHJlbmV3YWJsZSBlbmVyZ3ksIGltcHJvdmluZyB3YXRlciB1c2UgYW5kIHByb2R1Y3Rpdml0eS4gSW4gUGFraXN0YW4sIHdoZXJlIGFncmljdWx0dXJlIGNvbnRyaWJ1dGVzIDE5JSBvZiBncm9zcyBkb21lc3RpYyBwcm9kdWN0IGFuZCBlbXBsb3lzIDQwJSBvZiB0aGUgd29ya2ZvcmNlLCB0aGVzZSBjaGFsbGVuZ2VzIGFyZSBzZXZlcmUsIGVzcGVjaWFsbHkgaW4gd2F0ZXItc2NhcmNlIGFyZWFzIGxpa2UgdGhlIENob2xpc3RhbiBEZXNlcnQuIFRoaXMgc3R1ZHkgZXhhbWluZXMgdGhlIGltcGFjdCBvZiBzb2xhci1wb3dlcmVkIHNtYXJ0IGlycmlnYXRpb24gb24gYWdyaWN1bHR1cmFsIHByb2R1Y3Rpdml0eSwgd2F0ZXIgY29uc2VydmF0aW9uLCBhbmQgZW5lcmd5IGVmZmljaWVuY3kgaW4gdGhlIENob2xpc3RhbiBEZXNlcnQuIFVzaW5nIGEgcXVhbnRpdGF0aXZlIGNyb3NzLXNlY3Rpb25hbCBkZXNpZ24sIGRhdGEgd2VyZSBjb2xsZWN0ZWQgZnJvbSAzODQgZmFybWVycyB2aWEgc3RydWN0dXJlZCBxdWVzdGlvbm5haXJlcy4gU3RhdGlzdGljYWwgYW5hbHlzZXMsIGluY2x1ZGluZyBtdWx0aXBsZSBsaW5lYXIgcmVncmVzc2lvbiwgcGFpcmVkIHNhbXBsZSB0LXRlc3RzLCBhbmQgU3RydWN0dXJhbCBFcXVhdGlvbiBNb2RlbGluZyAoU0VNKSwgd2VyZSBjb25kdWN0ZWQuIEZpbmRpbmdzIHNob3cgc2lnbmlmaWNhbnQgaW1wcm92ZW1lbnRzIGluIGNyb3AgeWllbGQgKGZyb20gMy4wIHRvIDQuOCB0b25zL2hlY3RhcmUpIGFuZCByZWR1Y3Rpb25zIGluIHdhdGVyIGFuZCBlbmVyZ3kgY29uc3VtcHRpb24uIFJlZ3Jlc3Npb24gYW5hbHlzaXMgaGlnaGxpZ2h0ZWQgc3Ryb25nIHBvc2l0aXZlIGVmZmVjdHMgb24geWllbGQgYW5kIGVmZmljaWVuY3ksIHdoaWxlIFNFTSBjb25maXJtZWQgcmVkdWNlZCBlbnZpcm9ubWVudGFsIGltcGFjdCBhbmQgb3BlcmF0aW9uYWwgY29zdHMuIFRoZSBzdHVkeSBjb25jbHVkZXMgdGhhdCBzb2xhci1wb3dlcmVkIGlycmlnYXRpb24gZW5oYW5jZXMgcHJvZHVjdGl2aXR5LCBjb25zZXJ2ZXMgcmVzb3VyY2VzLCBhbmQgcHJvbW90ZXMgc3VzdGFpbmFiaWxpdHkuIFBvbGljeW1ha2VycyBzaG91bGQgcHJvdmlkZSBmaW5hbmNpYWwgaW5jZW50aXZlcywgaW52ZXN0IGluIHJlbmV3YWJsZSBpbmZyYXN0cnVjdHVyZSwgYW5kIGltcGxlbWVudCB0cmFpbmluZyBwcm9ncmFtcyB0byBzdXBwb3J0IGFkb3B0aW9uLiBDb2xsYWJvcmF0aXZlIGVmZm9ydHMgYXJlIGVzc2VudGlhbCBmb3Igc3VzdGFpbmFibGUgYWdyaWN1bHR1cmUgaW4gYXJpZCByZWdpb25zLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJpc3N1ZWREYXRlIjoiMjAyNS0wMy0wNSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkFncmljdWx0dXJlIiwiaXNzdWUiOjUsInZvbHVtZSI6MTUsImltcGFjdEZhY3RvciI6IjUuMjYifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjo1NTQsImxhc3RQYWdlIjo1NTR9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiTXVsdGlkaXNjaXBsaW5hcnkgRGlnaXRhbCBQdWJsaXNoaW5nIEluc3RpdHV0ZSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NDgsImNpdGVkQnlDb3VudCI6Mn19LCJxdW90ZSI6IjEuIEludHJvZHVjdGlvbiBUaGUgYWdyaWN1bHR1cmFsIHNlY3RvciwgYSBjb3JuZXJzdG9uZSBvZiBnbG9iYWwgZm9vZCBzZWN1cml0eSBhbmQgZWNvbm9taWMgZGV2ZWxvcG1lbnQsIGZhY2VzIG1vdW50aW5nIGNoYWxsZW5nZXMgaW4gdGhlIDIxc3QgY2VudHVyeS4gQ2xpbWF0ZSBjaGFuZ2UsIHJhcGlkIHBvcHVsYXRpb24gZ3Jvd3RoLCBhbmQgZHdpbmRsaW5nIG5hdHVyYWwgcmVzb3VyY2VzIGhhdmUgaW50ZW5zaWZpZWQgdGhlIG5lZWQgZm9yIHN1c3RhaW5hYmxlIGFncmljdWx0dXJhbCBwcmFjdGljZXMuIEFtb25nIHRoZXNlIGNoYWxsZW5nZXMsIHdhdGVyIHNjYXJjaXR5IGFuZCBlbmVyZ3kgaW5lZmZpY2llbmN5IHN0YW5kIG91dCBhcyBjcml0aWNhbCBiYXJyaWVycyB0byBwcm9kdWN0aXZpdHkgYW5kIHN1c3RhaW5hYmlsaXR5LiBHbG9iYWxseSwgYWdyaWN1bHR1cmUgYWNjb3VudHMgZm9yIGFwcHJveGltYXRlbHkgNzAlIG9mIGZyZXNod2F0ZXIgd2l0aGRyYXdhbHMgYW5kIGlzIG9uZSBvZiB0aGUgbGFyZ2VzdCBjb25zdW1lcnMgb2YgZW5lcmd5IGluIG1hbnkgcmVnaW9ucyBbIDEsIDJdLiBUcmFkaXRpb25hbCBpcnJpZ2F0aW9uIHByYWN0aWNlcywgb2Z0ZW4gaW5lZmZpY2llbnQgYW5kIHJlc291cmNlLWludGVuc2l2ZSwgZXhhY2VyYmF0ZSB0aGVzZSBpc3N1ZXMsIGxlYWRpbmcgdG8gd2F0ZXIgd2FzdGFnZSwgZW5lcmd5IG92ZXJ1c2UsIGFuZCByZWR1Y2VkIGNyb3AgeWllbGRzLiJ9XQ%3D%3D. This strategy is pivotal for smallholders facing erratic rainfall, complementing efficient delivery methods to curb agriculture's 70% freshwater dominance (Daraz et al., 2025)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlVtYXIiLCJmYW1pbHkiOiJEYXJheiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiLFoHRlZmFuIiwiZmFtaWx5IjoiQm9qbmVjIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllvdW5hcyIsImZhbWlseSI6IktoYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMDc3LTA0NzIiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzOTAvYWdyaWN1bHR1cmUxNTA1MDU1NCIsIm9wZW5hbGV4IjoiVzQ0MDgyMTk4MTEiLCJleGEiOiJodHRwczovL3d3dy5tZHBpLmNvbS8yMDc3LTA0NzIvMTUvNS81NTQifSwidGl0bGUiOiJFbmVyZ3ktRWZmaWNpZW50IFNtYXJ0IElycmlnYXRpb24gVGVjaG5vbG9naWVzOiBBIFBhdGh3YXkgdG8gV2F0ZXIgYW5kIEVuZXJneSBTdXN0YWluYWJpbGl0eSBpbiBBZ3JpY3VsdHVyZSIsImFic3RyYWN0IjoiVGhlIGFncmljdWx0dXJhbCBzZWN0b3IgZmFjZXMgY2hhbGxlbmdlcyBzdWNoIGFzIHdhdGVyIHNjYXJjaXR5LCBlbmVyZ3kgaW5lZmZpY2llbmN5LCBhbmQgZGVjbGluaW5nIHByb2R1Y3Rpdml0eSwgcGFydGljdWxhcmx5IGluIGFyaWQgcmVnaW9ucy4gVHJhZGl0aW9uYWwgaXJyaWdhdGlvbiBtZXRob2RzIGNvbnRyaWJ1dGUgdG8gcmVzb3VyY2UgZGVwbGV0aW9uIGFuZCBlbnZpcm9ubWVudGFsIGltcGFjdHMuIFNvbGFyLXBvd2VyZWQgc21hcnQgaXJyaWdhdGlvbiBzeXN0ZW1zIGludGVncmF0ZSBwcmVjaXNpb24gaXJyaWdhdGlvbiB3aXRoIHJlbmV3YWJsZSBlbmVyZ3ksIGltcHJvdmluZyB3YXRlciB1c2UgYW5kIHByb2R1Y3Rpdml0eS4gSW4gUGFraXN0YW4sIHdoZXJlIGFncmljdWx0dXJlIGNvbnRyaWJ1dGVzIDE5JSBvZiBncm9zcyBkb21lc3RpYyBwcm9kdWN0IGFuZCBlbXBsb3lzIDQwJSBvZiB0aGUgd29ya2ZvcmNlLCB0aGVzZSBjaGFsbGVuZ2VzIGFyZSBzZXZlcmUsIGVzcGVjaWFsbHkgaW4gd2F0ZXItc2NhcmNlIGFyZWFzIGxpa2UgdGhlIENob2xpc3RhbiBEZXNlcnQuIFRoaXMgc3R1ZHkgZXhhbWluZXMgdGhlIGltcGFjdCBvZiBzb2xhci1wb3dlcmVkIHNtYXJ0IGlycmlnYXRpb24gb24gYWdyaWN1bHR1cmFsIHByb2R1Y3Rpdml0eSwgd2F0ZXIgY29uc2VydmF0aW9uLCBhbmQgZW5lcmd5IGVmZmljaWVuY3kgaW4gdGhlIENob2xpc3RhbiBEZXNlcnQuIFVzaW5nIGEgcXVhbnRpdGF0aXZlIGNyb3NzLXNlY3Rpb25hbCBkZXNpZ24sIGRhdGEgd2VyZSBjb2xsZWN0ZWQgZnJvbSAzODQgZmFybWVycyB2aWEgc3RydWN0dXJlZCBxdWVzdGlvbm5haXJlcy4gU3RhdGlzdGljYWwgYW5hbHlzZXMsIGluY2x1ZGluZyBtdWx0aXBsZSBsaW5lYXIgcmVncmVzc2lvbiwgcGFpcmVkIHNhbXBsZSB0LXRlc3RzLCBhbmQgU3RydWN0dXJhbCBFcXVhdGlvbiBNb2RlbGluZyAoU0VNKSwgd2VyZSBjb25kdWN0ZWQuIEZpbmRpbmdzIHNob3cgc2lnbmlmaWNhbnQgaW1wcm92ZW1lbnRzIGluIGNyb3AgeWllbGQgKGZyb20gMy4wIHRvIDQuOCB0b25zL2hlY3RhcmUpIGFuZCByZWR1Y3Rpb25zIGluIHdhdGVyIGFuZCBlbmVyZ3kgY29uc3VtcHRpb24uIFJlZ3Jlc3Npb24gYW5hbHlzaXMgaGlnaGxpZ2h0ZWQgc3Ryb25nIHBvc2l0aXZlIGVmZmVjdHMgb24geWllbGQgYW5kIGVmZmljaWVuY3ksIHdoaWxlIFNFTSBjb25maXJtZWQgcmVkdWNlZCBlbnZpcm9ubWVudGFsIGltcGFjdCBhbmQgb3BlcmF0aW9uYWwgY29zdHMuIFRoZSBzdHVkeSBjb25jbHVkZXMgdGhhdCBzb2xhci1wb3dlcmVkIGlycmlnYXRpb24gZW5oYW5jZXMgcHJvZHVjdGl2aXR5LCBjb25zZXJ2ZXMgcmVzb3VyY2VzLCBhbmQgcHJvbW90ZXMgc3VzdGFpbmFiaWxpdHkuIFBvbGljeW1ha2VycyBzaG91bGQgcHJvdmlkZSBmaW5hbmNpYWwgaW5jZW50aXZlcywgaW52ZXN0IGluIHJlbmV3YWJsZSBpbmZyYXN0cnVjdHVyZSwgYW5kIGltcGxlbWVudCB0cmFpbmluZyBwcm9ncmFtcyB0byBzdXBwb3J0IGFkb3B0aW9uLiBDb2xsYWJvcmF0aXZlIGVmZm9ydHMgYXJlIGVzc2VudGlhbCBmb3Igc3VzdGFpbmFibGUgYWdyaWN1bHR1cmUgaW4gYXJpZCByZWdpb25zLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJpc3N1ZWREYXRlIjoiMjAyNS0wMy0wNSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkFncmljdWx0dXJlIiwiaXNzdWUiOjUsInZvbHVtZSI6MTUsImltcGFjdEZhY3RvciI6IjUuMjYifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjo1NTQsImxhc3RQYWdlIjo1NTR9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiTXVsdGlkaXNjaXBsaW5hcnkgRGlnaXRhbCBQdWJsaXNoaW5nIEluc3RpdHV0ZSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NDgsImNpdGVkQnlDb3VudCI6Mn19LCJxdW90ZSI6IjEuIEludHJvZHVjdGlvbiBUaGUgYWdyaWN1bHR1cmFsIHNlY3RvciwgYSBjb3JuZXJzdG9uZSBvZiBnbG9iYWwgZm9vZCBzZWN1cml0eSBhbmQgZWNvbm9taWMgZGV2ZWxvcG1lbnQsIGZhY2VzIG1vdW50aW5nIGNoYWxsZW5nZXMgaW4gdGhlIDIxc3QgY2VudHVyeS4gQ2xpbWF0ZSBjaGFuZ2UsIHJhcGlkIHBvcHVsYXRpb24gZ3Jvd3RoLCBhbmQgZHdpbmRsaW5nIG5hdHVyYWwgcmVzb3VyY2VzIGhhdmUgaW50ZW5zaWZpZWQgdGhlIG5lZWQgZm9yIHN1c3RhaW5hYmxlIGFncmljdWx0dXJhbCBwcmFjdGljZXMuIEFtb25nIHRoZXNlIGNoYWxsZW5nZXMsIHdhdGVyIHNjYXJjaXR5IGFuZCBlbmVyZ3kgaW5lZmZpY2llbmN5IHN0YW5kIG91dCBhcyBjcml0aWNhbCBiYXJyaWVycyB0byBwcm9kdWN0aXZpdHkgYW5kIHN1c3RhaW5hYmlsaXR5LiBHbG9iYWxseSwgYWdyaWN1bHR1cmUgYWNjb3VudHMgZm9yIGFwcHJveGltYXRlbHkgNzAlIG9mIGZyZXNod2F0ZXIgd2l0aGRyYXdhbHMgYW5kIGlzIG9uZSBvZiB0aGUgbGFyZ2VzdCBjb25zdW1lcnMgb2YgZW5lcmd5IGluIG1hbnkgcmVnaW9ucyBbIDEsIDJdLiBUcmFkaXRpb25hbCBpcnJpZ2F0aW9uIHByYWN0aWNlcywgb2Z0ZW4gaW5lZmZpY2llbnQgYW5kIHJlc291cmNlLWludGVuc2l2ZSwgZXhhY2VyYmF0ZSB0aGVzZSBpc3N1ZXMsIGxlYWRpbmcgdG8gd2F0ZXIgd2FzdGFnZSwgZW5lcmd5IG92ZXJ1c2UsIGFuZCByZWR1Y2VkIGNyb3AgeWllbGRzLiJ9XQ%3D%3D.

Fig 7- Mechanism of Subsburace Drip Irrigation (SDI)

Principles of Operation

Micro-catchments and rooftop systems direct water to lined reservoirs, filtered for quality; gravity-fed distribution minimizes energy (Kushwaha et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjMwLCJzdWJ0eXBlIjp7fSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTWFuaXNoIiwiZmFtaWx5IjoiS3VzaHdhaGEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2hhbmthciIsImZhbWlseSI6IlNpbmdoIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlZpamF5IEthbnQiLCJmYW1pbHkiOiJTaW5naCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTaGFzaGFuayIsImZhbWlseSI6IkR3aXZlZGkifX1dLCJpZHMiOnsiaXNzbiI6WyIyNDU2LTE4NzgiXSwiaXNibiI6W10sImRvaSI6IjEwLjIyMTYxL2lqZWFiLjkyLjI3Iiwib3BlbmFsZXgiOiJXNDM5NjU2MzU4NCJ9LCJ0aXRsZSI6IlByZWNpc2lvbiBGYXJtaW5nOiBBIFJldmlldyBvZiBNZXRob2RzLCBUZWNobm9sb2dpZXMsIGFuZCBGdXR1cmUgUHJvc3BlY3RzIiwiYWJzdHJhY3QiOiJQcmVjaXNpb24gZmFybWluZywgb2Z0ZW4gcmVmZXJyZWQgdG8gYXMgcHJlY2lzaW9uIGFncmljdWx0dXJlLCBpcyBhIGZhcm1pbmcgbWFuYWdlbWVudCBjb25jZXB0IHRoYXQgc2Vla3MgdG8gbWF4aW1pemUgYWdyaWN1bHR1cmFsIG9wZXJhdGlvbnMgdmlhIHRoZSB1c2Ugb2YgdGVjaG5vbG9neSBpbiBvcmRlciB0byBhY2hpZXZlIGdyZWF0ZXIgbGV2ZWxzIG9mIGVmZmljaWVuY3ksIHByb2R1Y3Rpb24sIGFuZCBjb25zZXJ2YXRpb24gb2YgcmVzb3VyY2VzLiBUaGUgcHVycG9zZSBvZiB0aGlzIHN1cnZleSBzdHVkeSBpcyB0byBvZmZlciBhbiBvdmVydmlldyBvZiBwcmVjaXNpb24gZmFybWluZywgaW5jbHVkaW5nIHRvcGljcyBzdWNoIGFzIGl0cyBmdW5kYW1lbnRhbHMsIG1ldGhvZG9sb2dpZXMsIHRlY2hub2xvZ3ksIGFuZCBhcHBsaWNhdGlvbnMuIEl0IGludmVzdGlnYXRlcyB0aGUgcm9sZSB0aGF0IGEgdmFyaWV0eSBvZiB0ZWNobm9sb2dpZXMsIGluY2x1ZGluZyBnbG9iYWwgbmF2aWdhdGlvbiBzYXRlbGxpdGUgc3lzdGVtcyAoR05TUyksIGdlb2dyYXBoaWMgaW5mb3JtYXRpb24gc3lzdGVtcyAoR0lTKSwgcmVtb3RlIHNlbnNpbmcsIGFuZCBzZW5zb3IgdGVjaG5vbG9neSwgcGxheSBpbiBtYWtpbmcgaXQgcG9zc2libGUgdG8gaW1wbGVtZW50IHByZWNpc2lvbiBhZ3JpY3VsdHVyYWwgbWV0aG9kcy4gSW4gYWRkaXRpb24sIGl0IGFkZHJlc3NlcyB0aGUgYWR2YW50YWdlcywgZGlzYWR2YW50YWdlcywgYW5kIHBvdGVudGlhbCBmdXR1cmUgYXBwbGljYXRpb25zIG9mIHByZWNpc2lvbiBmYXJtaW5nIGluIHRoZSBjb250ZXh0IG9mIHNvbHZpbmcgaXNzdWVzIHJlbGF0ZWQgdG8gdGhlIHN1c3RhaW5hYmlsaXR5IG9mIGFncmljdWx0dXJlIGFuZCB0aGUgc2VjdXJpdHkgb2YgZm9vZCBzdXBwbGllcyBvbiBhIHdvcmxkd2lkZSBzY2FsZS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjIxNjEvaWplYWIuOTIuMjciLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vaWplYWIuY29tL3VwbG9hZF9kb2N1bWVudC9pc3N1ZV9maWxlcy8yN0lKRUFCLTEwNDIwMjQ4NC1QcmVjaXNpb25GYXJtaW5nLnBkZiIsImlzc3VlZERhdGUiOiIyMDI0LTAxLTAxIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiSW50ZXJuYXRpb25hbCBKb3VybmFsIG9mIEVudmlyb25tZW50IEFncmljdWx0dXJlIGFuZCBCaW90ZWNobm9sb2d5IiwiaXNzdWUiOjIsInZvbHVtZSI6OSwiaW1wYWN0RmFjdG9yIjoiMC42NSJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjI0MiwibGFzdFBhZ2UiOjI1M319LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJJbmZvZ2FpbiBQdWJsaWNhdGlvbiJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NCwiY2l0ZWRCeUNvdW50Ijo4fX0sInF1b3RlIjoiSUlJLiBCRU5FRklUUyBPRiBQUkVDSVNJT04gSVJSSUdBVElPTiAxLiBJbXByb3ZlZCBXYXRlciBVc2UgRWZmaWNpZW5jeTogUHJlY2lzaW9uIGlycmlnYXRpb24gb3B0aW1pemVzIHdhdGVyIHVzZSBlZmZpY2llbmN5IGJ5IGRlbGl2ZXJpbmcgd2F0ZXIgZGlyZWN0bHkgdG8gdGhlIHJvb3Qgem9uZSBvZiBjcm9wcywgbWluaW1pemluZyBsb3NzZXMgIGR1ZSB0byBldmFwb3JhdGlvbiwgcnVub2ZmLCBhbmQgZGVlcCBwZXJjb2xhdGlvbi4gQnkgIG1hdGNoaW5nIHdhdGVyIGFwcGxpY2F0aW9uIHJhdGVzIHRvIGNyb3Agd2F0ZXIgIHJlcXVpcmVtZW50cywgcHJlY2lzaW9uIGlycmlnYXRpb24gc3lzdGVtcyBlbnN1cmUgdGhhdCAgd2F0ZXIgaXMgdXNlZCBtb3JlIGVmZmljaWVudGx5LCBtYXhpbWl6aW5nIGNyb3AgeWllbGRzICBwZXIgdW5pdCBvZiB3YXRlciBhcHBsaWVkLiAyLiBFbmhhbmNlZCBDcm9wIFBlcmZvcm1hbmNlOiBCeSBwcm92aWRpbmcgY3JvcHMgd2l0aCBvcHRpbWFsIG1vaXN0dXJlIGxldmVscywgcHJlY2lzaW9uIGlycmlnYXRpb24gc3lzdGVtcyBwcm9tb3RlIGhlYWx0aHkgcm9vdCBkZXZlbG9wbWVudCwgYmFsYW5jZWQgZ3Jvd3RoLCAgYW5kIGltcHJvdmVkIGNyb3Agdmlnb3IuIENvbnNpc3RlbnQgc29pbCBtb2lzdHVyZSAgbGV2ZWxzIGhlbHAgcmVkdWNlIHdhdGVyIHN0cmVzcywgaW5jcmVhc2UgbnV0cmllbnQgIHVwdGFrZSwgYW5kIGVuaGFuY2UgY3JvcCByZXNpbGllbmNlIHRvIGVudmlyb25tZW50YWwgIHN0cmVzc2VzIHN1Y2ggYXMgZHJvdWdodCBhbmQgaGVhdC4gMy4gQ29zdCBTYXZpbmdzOiBQcmVjaXNpb24gaXJyaWdhdGlvbiBzeXN0ZW1zIGhlbHAgZmFybWVycyByZWR1Y2Ugd2F0ZXIsIGVuZXJneSwgYW5kIGxhYm9yIGNvc3RzIGFzc29jaWF0ZWQgd2l0aCBpcnJpZ2F0aW9uLiBCeSBvcHRpbWl6aW5nIHdhdGVyIHVzZSBlZmZpY2llbmN5IGFuZCAgbWluaW1pemluZyBpbnB1dCB3YXN0YWdlLCBwcmVjaXNpb24gaXJyaWdhdGlvbiBzeXN0ZW1zICBjYW4gbGVhZCB0byBzaWduaWZpY2FudCBjb3N0IHNhdmluZ3Mgb3ZlciB0aW1lLCAgaW1wcm92aW5nIHRoZSBwcm9maXRhYmlsaXR5IG9mIGFncmljdWx0dXJhbCBvcGVyYXRpb25zLiA0LiBFbnZpcm9ubWVudGFsIFN1c3RhaW5hYmlsaXR5OiAgUHJlY2lzaW9uICBpcnJpZ2F0aW9uIGNvbnRyaWJ1dGVzIHRvICBlbnZpcm9ubWVudGFsICBzdXN0YWluYWJpbGl0eSAgYnkgY29uc2VydmluZyB3YXRlciAgcmVzb3VyY2VzLCAgbWluaW1pemluZyAgd2F0ZXIgcG9sbHV0aW9uLCBhbmQgcmVkdWNpbmcgZ3JlZW5ob3VzZSBnYXMgZW1pc3Npb25zLiBCeSBhZG9wdGluZyBwcmVjaXNpb24gaXJyaWdhdGlvbiBwcmFjdGljZXMsIGZhcm1lcnMgY2FuICBtaW5pbWl6ZSB0aGVpciBlbnZpcm9ubWVudGFsIGZvb3RwcmludCBhbmQgY29udHJpYnV0ZSB0byB0aGUgIGxvbmctdGVybSAgc3VzdGFpbmFiaWxpdHkgIG9mICBhZ3JpY3VsdHVyYWwgcHJvZHVjdGlvbiBzeXN0ZW1zLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjQ0Ljg2NTY0MjU0NzYwNzQyLCJsZWZ0Ijo1My40MDgyNTI3MTYwNjQ0NSwid2lkdGgiOjM4LjI2MTc2ODM0MTA2NDQ1LCJoZWlnaHQiOjEuNjY5MjI3MzYxNjc5MDc3MX19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjQ2Ljg5NzkyMjUxNTg2OTE0LCJsZWZ0Ijo1Mi40MjA1NDM2NzA2NTQzLCJ3aWR0aCI6NDAuMjQwNzM3OTE1MDM5MDYsImhlaWdodCI6MS42MjU0NTQ3ODM0Mzk2MzYyfX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NDguNDkxNzkwNzcxNDg0Mzc1LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3Ljk4MDI3ODAxNTEzNjcyLCJoZWlnaHQiOjExLjAxMzcxMzgzNjY2OTkyMn19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjU5LjkzOTU0MDg2MzAzNzExLCJsZWZ0Ijo1Mi40MjA1NDM2NzA2NTQzLCJ3aWR0aCI6NDAuMjMwMjI0NjA5Mzc1LCJoZWlnaHQiOjEuNjI1NDU0NzgzNDM5NjM2Mn19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjYxLjUzMzQxNjc0ODA0Njg3NSwibGVmdCI6NTQuNjk4MzEwODUyMDUwNzgsIndpZHRoIjozNy45NjMxMjMzMjE1MzMyLCJoZWlnaHQiOjkuNDM1MTcwMTczNjQ1MDJ9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo3MS40MTY4Nzc3NDY1ODIwMywibGVmdCI6NTIuNDIwNTQzNjcwNjU0Mywid2lkdGgiOjQwLjIxMzMxMDI0MTY5OTIyLCJoZWlnaHQiOjEuNjI1NDU4MzU5NzE4MzIyOH19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjczLjAxMDc1NzQ0NjI4OTA2LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3Ljk0NTc4NTUyMjQ2MDk0LCJoZWlnaHQiOjcuODYzNzY0Mjg2MDQxMjZ9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo4MS4zMDg1NTU2MDMwMjczNCwibGVmdCI6NTIuNDIwNTQzNjcwNjU0Mywid2lkdGgiOjQwLjI0MDM3NTUxODc5ODgzLCJoZWlnaHQiOjEuNjI1NDU4MzU5NzE4MzIyOH19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjgyLjkxOTA1OTc1MzQxNzk3LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3LjkzNjQxNjYyNTk3NjU2LCJoZWlnaHQiOjEuNTc4MTMzNTgzMDY4ODQ3N319LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjg0LjQ4NjkwMDMyOTU4OTg0LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3LjkzNDc1NzIzMjY2NjAxNiwiaGVpZ2h0IjoxLjU3ODEzMzU4MzA2ODg0Nzd9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo4Ni4wNTQ3NDg1MzUxNTYyNSwibGVmdCI6NTQuNjk4MzEwODUyMDUwNzgsIndpZHRoIjozNy45Nzg0MzE3MDE2NjAxNTYsImhlaWdodCI6NC43MTM4MTkwMjY5NDcwMjE1fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6OC40MjAyOTI4NTQzMDkwODIsImxlZnQiOjEwLjAyMjE3MjkyNzg1NjQ0NSwid2lkdGgiOjM3Ljk0NjgzMDc0OTUxMTcyLCJoZWlnaHQiOjEuNTc4MTM1MjUxOTk4OTAxNH19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjkuOTg4MTMyNDc2ODA2NjQsImxlZnQiOjEwLjAyMjE3MjkyNzg1NjQ0NSwid2lkdGgiOjEzLjg2MjQ2MTA5MDA4Nzg5LCJoZWlnaHQiOjEuNTc4MTM1MjUxOTk4OTAxNH19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjI0OSJ9fV0%3D.

Advantages and Efficiency Gains

- Supplemental supply: Fills gaps in drip/smart systems (Jumintono et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikp1bWludG9ubyIsImZhbWlseSI6Ikp1bWludG9ubyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJOdXJuYXdhdHkiLCJmYW1pbHkiOiJOdXJuYXdhdHkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTnVncm9obyBUcmlzbnUiLCJmYW1pbHkiOiJCcmF0YSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJEaW5pIiwiZmFtaWx5IjoiUmFobWlhdGkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSGFkaSIsImZhbWlseSI6IlN1c2lsbyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBLiBBLiIsImZhbWlseSI6Ikh1c3NlbiJ9fV0sImlkcyI6eyJpc3NuIjpbIjE0MjktNzQyNiIsIjIwODMtNDUzNSJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMjQ0MjUvandsZC4yMDI0LjE1MTc5OCIsIm9wZW5hbGV4IjoiVzQ0MDU0NzcxMDAifSwidGl0bGUiOiJDb21wYXJhdGl2ZSBhbmFseXNpcyBvZiBwcmVjaXNpb24sIGRlZmljaXQsIGFuZCBkcmlwIGlycmlnYXRpb24gZm9yIHdhdGVyIGVmZmljaWVuY3kgaW4gSXJhcSIsImFic3RyYWN0IjoiU3VzdGFpbmFibGUgaXJyaWdhdGlvbiBwcmFjdGljZXMgYXJlIGNyaXRpY2FsIGZvciBhZGRyZXNzaW5nIHRoZSBjaGFsbGVuZ2VzIG9mIHdhdGVyIHNjYXJjaXR5IGFuZCBmb29kIGluc2VjdXJpdHkgaW4gdGhlIGZhY2Ugb2YgY2xpbWF0ZSBjaGFuZ2UgYW5kIGdyb3dpbmcgZ2xvYmFsIGZvb2QgZGVtYW5kLiBUaGlzIHN0dWR5IGludmVzdGlnYXRlZCB0aGUgcG90ZW50aWFsIG9mIHByZWNpc2lvbiBpcnJpZ2F0aW9uIChQSSksIGRlZmljaXQgaXJyaWdhdGlvbiAoREkpLCBhbmQgZHJpcCBpcnJpZ2F0aW9uIChEUkkpIHRlY2huaXF1ZXMgdG8gZW5oYW5jZSBjcm9wIHByb2R1Y3Rpdml0eSBhbmQgd2F0ZXIgdXNlIGVmZmljaWVuY3kgKFdVRSkgY29tcGFyZWQgdG8gY29udmVudGlvbmFsIGZsb29kIGlycmlnYXRpb24gKENGSSkgaW4gdGhyZWUgZGlzdGluY3QgYWdyb2Vjb2xvZ2ljYWwgem9uZXMgb2YgSXJhcS4gRmllbGQgZXhwZXJpbWVudHMgd2VyZSBjb25kdWN0ZWQgdXNpbmcgYSByYW5kb21pc2VkIGNvbXBsZXRlIGJsb2NrIGRlc2lnbiB3aXRoIHdoZWF0LCBtYWl6ZSwgYW5kIHJpY2UgY3JvcHMuIFRoZSByZXN1bHRzIGRlbW9uc3RyYXRlZCB0aGF0IFBJIHNpZ25pZmljYW50bHkgaW5jcmVhc2VkIGNyb3AgZ3Jvd3RoIHBhcmFtZXRlcnMsIGdyYWluIHlpZWxkLCBhbmQgV1VFIGFjcm9zcyBhbGwgem9uZXMsIHdpdGggeWllbGQgaW1wcm92ZW1lbnRzIG9mIDMz4oCTMzglIGFuZCBXVUUgaW5jcmVhc2VzIG9mIDQ24oCTNTElIGluIGNvbnRyYXN0IHdpdGggQ0ZJLiBUaGUgREkgYW5kIERSSSB0cmVhdG1lbnRzIGFsc28gb3V0cGVyZm9ybWVkIENGSSwgYWxiZWl0IHRvIGEgbGVzc2VyIGV4dGVudC4gUmVtb3RlIHNlbnNpbmctZGVyaXZlZCB2ZWdldGF0aW9uIGluZGljZXMgc3Ryb25nbHkgY29ycmVsYXRlZCB3aXRoIGNyb3AgZ3Jvd3RoIHBhcmFtZXRlcnMgYW5kIHlpZWxkLCB3aGlsZSBoeWRyb2xvZ2ljYWwgbW9kZWxsaW5nIHJldmVhbGVkIHJlZHVjZWQgZXZhcG90cmFuc3BpcmF0aW9uIGFuZCBzdXJmYWNlIHJ1bm9mZiB1bmRlciB0aGUgUEkgdHJlYXRtZW50LiBUaGUgc3VzdGFpbmFibGUgaXJyaWdhdGlvbiBwcmFjdGljZXMgcmVzdWx0ZWQgaW4gc3Vic3RhbnRpYWwgd2F0ZXIgc2F2aW5ncyBvZiAyMOKAkzMwJSBjb21wYXJlZCB0byBDRkkuIFRoZXNlIGZpbmRpbmdzIGhpZ2hsaWdodCB0aGUgaW1wb3J0YW5jZSBvZiBhZG9wdGluZyBlZmZpY2llbnQgaXJyaWdhdGlvbiB0ZWNobmlxdWVzLCBhbG9uZyB3aXRoIGEgaG9saXN0aWMgYXBwcm9hY2ggZW5jb21wYXNzaW5nIHRlY2hub2xvZ2ljYWwgaW5ub3ZhdGlvbnMsIGNhcGFjaXR5IGJ1aWxkaW5nLCBhbmQgc3Rha2Vob2xkZXIgZW5nYWdlbWVudCwgdG8gcHJvbW90ZSB3YXRlci0gZWZmaWNpZW50IGFncmljdWx0dXJlIGFuZCBlbnN1cmUgZm9vZCBzZWN1cml0eSBpbiB3YXRlci1zY2FyY2UgcmVnaW9ucy4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjQ0MjUvandsZC4yMDI0LjE1MTc5OCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjI0NDI1L2p3bGQuMjAyNC4xNTE3OTgiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMi0xNCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkpvdXJuYWwgb2YgV2F0ZXIgYW5kIExhbmQgRGV2ZWxvcG1lbnQiLCJpbXBhY3RGYWN0b3IiOiIxLjQ2In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTMwLCJsYXN0UGFnZSI6MTM2fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkRlIEdydXl0ZXIgT3BlbiJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MzIsImNpdGVkQnlDb3VudCI6MX19LCJxdW90ZSI6IkluIHRoaXMgY29udGV4dCwgdGhlIGFkb3B0aW9uIG9mICBzdXN0YWluYWJsZSBpcnJpZ2F0aW9uIHByYWN0aWNlcyBoYXMgZW1lcmdlZCBhcyBhIGNyaXRpY2FsIHN0cmF0ZWd5ICBmb3IgZW5oYW5jaW5nIGZvb2Qgc2VjdXJpdHkgd2hpbGUgcHJvbW90aW5nIHdhdGVyIGNvbnNlcnZhdGlvbiAgaW4gYWdyaWN1bHR1cmFsIGxhbmRzY2FwZXMuICBJcnJpZ2F0aW9uIHBsYXlzIGEgdml0YWwgcm9sZSBpbiBnbG9iYWwgZm9vZCBwcm9kdWN0aW9uLCB3aXRoICBpcnJpZ2F0ZWQgYWdyaWN1bHR1cmUgYWNjb3VudGluZyBmb3IgYXBwcm94aW1hdGVseSA0MCUgb2YgdGhlICBmb29kIHN1cHBseSAoV3JhY2hpZW4gZGUsIFNjaHVsdHogYW5kIEdvbGksIDIwMjE7IFNjaG1pdHQsIFJvc2EgIGFuZCBEYWlseSwgMjAyMikuIEhvd2V2ZXIsIHRyYWRpdGlvbmFsIGlycmlnYXRpb24gbWV0aG9kcywgc3VjaCBhcyAgZmxvb2QgYW5kIGZ1cnJvdyBpcnJpZ2F0aW9uLCBhcmUgb2Z0ZW4gaW5lZmZpY2llbnQsIGxlYWRpbmcgdG8gIHN1YnN0YW50aWFsIHdhdGVyIGxvc3NlcyB0aHJvdWdoIGV2YXBvcmF0aW9uLCBydW5vZmYsIGFuZCBkZWVwICBwZXJjb2xhdGlvbiAoQW1ib21zYSwgMjAyMCkuIE1vcmVvdmVyLCB0aGUgZXhjZXNzaXZlIGFwcGxpY2EtdGlvbiBvZiB3YXRlciBjYW4gcmVzdWx0IGluIHNvaWwgc2FsaW5pc2F0aW9uLCBudXRyaWVudCBsZWFjaGluZywgYW5kICBncm91bmR3YXRlciBkZXBsZXRpb24sIGZ1cnRoZXIgY29tcHJvbWlzaW5nIHRoZSBsb25nLXRlcm0gIHN1c3RhaW5hYmlsaXR5IG9mIGFncmljdWx0dXJhbCBzeXN0ZW1zIChZaW4gZXQgYWwuLCAyMDIyKS4gVG8gYWRkcmVzcyAgdGhlc2UgY2hhbGxlbmdlcywgcmVzZWFyY2hlcnMgYW5kIHByYWN0aXRpb25lcnMgaGF2ZSBiZWVuIGV4cGxvci1pbmcgaW5ub3ZhdGl2ZSBpcnJpZ2F0aW9uIHRlY2huaXF1ZXMgYW5kIHdhdGVyIG1hbmFnZW1lbnQgIHN0cmF0ZWdpZXMgdGhhdCBjYW4gb3B0aW1pc2Ugd2F0ZXIgdXNlIGVmZmljaWVuY3kgKFdVRSkgd2hpbGUgIG1haW50YWluaW5nIG9yIGVuaGFuY2luZyBjcm9wIHByb2R1Y3Rpdml0eS4gIFByZWNpc2lvbiBpcnJpZ2F0aW9uIChQSSkgb2ZmZXJzIGEgcHJvbWlzaW5nIHNvbHV0aW9uLCAgaW52b2x2aW5nIHRoZSB0YXJnZXRlZCBhcHBsaWNhdGlvbiBvZiB3YXRlciBzcGVjaWZpY2FsbHkgdG8gdGhlICBuZWVkcyBvZiBpbmRpdmlkdWFsIHBsYW50cyBvciBjcm9wIHpvbmVzIChaaW5rZXJuYWdlbCBldCBhbC4sIDIwMjApLiAgIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyMi44ODY3MzU5MTYxMzc2OTUsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzQ2MjIxOTIzODI4MTI1LCJoZWlnaHQiOjEuNDUwNTQxMDE5NDM5Njk3M319LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI0LjM1MTEyMzgwOTgxNDQ1MywibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTI3MTYwNjQ0NSwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI1LjcyNDk4NzAzMDAyOTI5NywibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTY1MzA3NjE3MiwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI3LjA5ODg1MDI1MDI0NDE0LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjE2LjQxODE2MTM5MjIxMTkxNCwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI4LjQ3MjcwOTY1NTc2MTcyLCJsZWZ0IjoxMi4zODA4ODAzNTU4MzQ5NjEsIndpZHRoIjozNy4wMTIxODQxNDMwNjY0MDYsImhlaWdodCI6MS4zNjAwMTQ3OTYyNTcwMTl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyOS44NDY1NzI4NzU5NzY1NjIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQwNjQ4NjUxMTIzMDUsImhlaWdodCI6MS4zNjAwMTY3MDM2MDU2NTE5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzEuMjE0MDM2OTQxNTI4MzIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQ0ODU2MjYyMjA3MDMsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzIuNTg3OTAyMDY5MDkxOCwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDYwNTc4OTE4NDU3LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMzLjk2MTc2MTQ3NDYwOTM3NSwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDA0ODQ2MTkxNDA2MjUsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzUuMzM1NjI0Njk0ODI0MjIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzM5NzQ0NTY3ODcxMDk0LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjM2LjcwOTQ4NzkxNTAzOTA2LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjAwMTU5MDcyODc1OTc2NiwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC4wODMzNDczMjA1NTY2NCwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTY1MzA3NjE3MiwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOS40NTcyMTA1NDA3NzE0ODQsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQzMDQ4MDk1NzAzMTI1LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQwLjc0MDU0NzE4MDE3NTc4LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjM0NTQ1NTE2OTY3NzczNCwiaGVpZ2h0IjoxLjQ1MDU0MjgwNzU3OTA0MDV9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Mi4yMDQ5MzMxNjY1MDM5MDYsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMDAzMjUwMTIyMDcwMzEsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDMuNTc4Nzk2Mzg2NzE4NzUsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzM3OTIxMTQyNTc4MTI1LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQ0Ljg2MjEzMzAyNjEyMzA1LCJsZWZ0Ijo5LjA0NzYwMTY5OTgyOTEwMiwid2lkdGgiOjQwLjM0NTMxNDAyNTg3ODkwNiwiaGVpZ2h0IjoxLjQ1MDU0MjgwNzU3OTA0MDV9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ni4zMjY1MTkwMTI0NTExNywibGVmdCI6OS4wNDc2MDE2OTk4MjkxMDIsIndpZHRoIjoyNy45Njc0NDcyODA4ODM3OSwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ny43MDAzODIyMzI2NjYwMTYsImxlZnQiOjEyLjM4MDg3ODQ0ODQ4NjMyOCwid2lkdGgiOjM3LjAzNDc4NjIyNDM2NTIzNCwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0OS4wNzQyNDU0NTI4ODA4NiwibGVmdCI6OS4wNDc2MDE2OTk4MjkxMDIsIndpZHRoIjo0MC4zNDA2NDQ4MzY0MjU3OCwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1MC4zNTc1ODIwOTIyODUxNTYsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzQ0ODUyNDQ3NTA5NzY2LCJoZWlnaHQiOjEuNDUwNTQyODA3NTc5MDQwNX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEzMSJ9fV0%3D;

- Cost-effective: Low-tech for adoption;

- Erosion control: Enhances soil moisture retention.

Applications and Implementation

Prevalent in semi-arid India, Africa; integrated with precision tech for optimal use (Daraz et al., 2025)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlVtYXIiLCJmYW1pbHkiOiJEYXJheiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiLFoHRlZmFuIiwiZmFtaWx5IjoiQm9qbmVjIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllvdW5hcyIsImZhbWlseSI6IktoYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMDc3LTA0NzIiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzOTAvYWdyaWN1bHR1cmUxNTA1MDU1NCIsIm9wZW5hbGV4IjoiVzQ0MDgyMTk4MTEiLCJleGEiOiJodHRwczovL3d3dy5tZHBpLmNvbS8yMDc3LTA0NzIvMTUvNS81NTQifSwidGl0bGUiOiJFbmVyZ3ktRWZmaWNpZW50IFNtYXJ0IElycmlnYXRpb24gVGVjaG5vbG9naWVzOiBBIFBhdGh3YXkgdG8gV2F0ZXIgYW5kIEVuZXJneSBTdXN0YWluYWJpbGl0eSBpbiBBZ3JpY3VsdHVyZSIsImFic3RyYWN0IjoiVGhlIGFncmljdWx0dXJhbCBzZWN0b3IgZmFjZXMgY2hhbGxlbmdlcyBzdWNoIGFzIHdhdGVyIHNjYXJjaXR5LCBlbmVyZ3kgaW5lZmZpY2llbmN5LCBhbmQgZGVjbGluaW5nIHByb2R1Y3Rpdml0eSwgcGFydGljdWxhcmx5IGluIGFyaWQgcmVnaW9ucy4gVHJhZGl0aW9uYWwgaXJyaWdhdGlvbiBtZXRob2RzIGNvbnRyaWJ1dGUgdG8gcmVzb3VyY2UgZGVwbGV0aW9uIGFuZCBlbnZpcm9ubWVudGFsIGltcGFjdHMuIFNvbGFyLXBvd2VyZWQgc21hcnQgaXJyaWdhdGlvbiBzeXN0ZW1zIGludGVncmF0ZSBwcmVjaXNpb24gaXJyaWdhdGlvbiB3aXRoIHJlbmV3YWJsZSBlbmVyZ3ksIGltcHJvdmluZyB3YXRlciB1c2UgYW5kIHByb2R1Y3Rpdml0eS4gSW4gUGFraXN0YW4sIHdoZXJlIGFncmljdWx0dXJlIGNvbnRyaWJ1dGVzIDE5JSBvZiBncm9zcyBkb21lc3RpYyBwcm9kdWN0IGFuZCBlbXBsb3lzIDQwJSBvZiB0aGUgd29ya2ZvcmNlLCB0aGVzZSBjaGFsbGVuZ2VzIGFyZSBzZXZlcmUsIGVzcGVjaWFsbHkgaW4gd2F0ZXItc2NhcmNlIGFyZWFzIGxpa2UgdGhlIENob2xpc3RhbiBEZXNlcnQuIFRoaXMgc3R1ZHkgZXhhbWluZXMgdGhlIGltcGFjdCBvZiBzb2xhci1wb3dlcmVkIHNtYXJ0IGlycmlnYXRpb24gb24gYWdyaWN1bHR1cmFsIHByb2R1Y3Rpdml0eSwgd2F0ZXIgY29uc2VydmF0aW9uLCBhbmQgZW5lcmd5IGVmZmljaWVuY3kgaW4gdGhlIENob2xpc3RhbiBEZXNlcnQuIFVzaW5nIGEgcXVhbnRpdGF0aXZlIGNyb3NzLXNlY3Rpb25hbCBkZXNpZ24sIGRhdGEgd2VyZSBjb2xsZWN0ZWQgZnJvbSAzODQgZmFybWVycyB2aWEgc3RydWN0dXJlZCBxdWVzdGlvbm5haXJlcy4gU3RhdGlzdGljYWwgYW5hbHlzZXMsIGluY2x1ZGluZyBtdWx0aXBsZSBsaW5lYXIgcmVncmVzc2lvbiwgcGFpcmVkIHNhbXBsZSB0LXRlc3RzLCBhbmQgU3RydWN0dXJhbCBFcXVhdGlvbiBNb2RlbGluZyAoU0VNKSwgd2VyZSBjb25kdWN0ZWQuIEZpbmRpbmdzIHNob3cgc2lnbmlmaWNhbnQgaW1wcm92ZW1lbnRzIGluIGNyb3AgeWllbGQgKGZyb20gMy4wIHRvIDQuOCB0b25zL2hlY3RhcmUpIGFuZCByZWR1Y3Rpb25zIGluIHdhdGVyIGFuZCBlbmVyZ3kgY29uc3VtcHRpb24uIFJlZ3Jlc3Npb24gYW5hbHlzaXMgaGlnaGxpZ2h0ZWQgc3Ryb25nIHBvc2l0aXZlIGVmZmVjdHMgb24geWllbGQgYW5kIGVmZmljaWVuY3ksIHdoaWxlIFNFTSBjb25maXJtZWQgcmVkdWNlZCBlbnZpcm9ubWVudGFsIGltcGFjdCBhbmQgb3BlcmF0aW9uYWwgY29zdHMuIFRoZSBzdHVkeSBjb25jbHVkZXMgdGhhdCBzb2xhci1wb3dlcmVkIGlycmlnYXRpb24gZW5oYW5jZXMgcHJvZHVjdGl2aXR5LCBjb25zZXJ2ZXMgcmVzb3VyY2VzLCBhbmQgcHJvbW90ZXMgc3VzdGFpbmFiaWxpdHkuIFBvbGljeW1ha2VycyBzaG91bGQgcHJvdmlkZSBmaW5hbmNpYWwgaW5jZW50aXZlcywgaW52ZXN0IGluIHJlbmV3YWJsZSBpbmZyYXN0cnVjdHVyZSwgYW5kIGltcGxlbWVudCB0cmFpbmluZyBwcm9ncmFtcyB0byBzdXBwb3J0IGFkb3B0aW9uLiBDb2xsYWJvcmF0aXZlIGVmZm9ydHMgYXJlIGVzc2VudGlhbCBmb3Igc3VzdGFpbmFibGUgYWdyaWN1bHR1cmUgaW4gYXJpZCByZWdpb25zLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJpc3N1ZWREYXRlIjoiMjAyNS0wMy0wNSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkFncmljdWx0dXJlIiwiaXNzdWUiOjUsInZvbHVtZSI6MTUsImltcGFjdEZhY3RvciI6IjUuMjYifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjo1NTQsImxhc3RQYWdlIjo1NTR9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiTXVsdGlkaXNjaXBsaW5hcnkgRGlnaXRhbCBQdWJsaXNoaW5nIEluc3RpdHV0ZSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NDgsImNpdGVkQnlDb3VudCI6Mn19LCJxdW90ZSI6IjEuIEludHJvZHVjdGlvbiBUaGUgYWdyaWN1bHR1cmFsIHNlY3RvciwgYSBjb3JuZXJzdG9uZSBvZiBnbG9iYWwgZm9vZCBzZWN1cml0eSBhbmQgZWNvbm9taWMgZGV2ZWxvcG1lbnQsIGZhY2VzIG1vdW50aW5nIGNoYWxsZW5nZXMgaW4gdGhlIDIxc3QgY2VudHVyeS4gQ2xpbWF0ZSBjaGFuZ2UsIHJhcGlkIHBvcHVsYXRpb24gZ3Jvd3RoLCBhbmQgZHdpbmRsaW5nIG5hdHVyYWwgcmVzb3VyY2VzIGhhdmUgaW50ZW5zaWZpZWQgdGhlIG5lZWQgZm9yIHN1c3RhaW5hYmxlIGFncmljdWx0dXJhbCBwcmFjdGljZXMuIEFtb25nIHRoZXNlIGNoYWxsZW5nZXMsIHdhdGVyIHNjYXJjaXR5IGFuZCBlbmVyZ3kgaW5lZmZpY2llbmN5IHN0YW5kIG91dCBhcyBjcml0aWNhbCBiYXJyaWVycyB0byBwcm9kdWN0aXZpdHkgYW5kIHN1c3RhaW5hYmlsaXR5LiBHbG9iYWxseSwgYWdyaWN1bHR1cmUgYWNjb3VudHMgZm9yIGFwcHJveGltYXRlbHkgNzAlIG9mIGZyZXNod2F0ZXIgd2l0aGRyYXdhbHMgYW5kIGlzIG9uZSBvZiB0aGUgbGFyZ2VzdCBjb25zdW1lcnMgb2YgZW5lcmd5IGluIG1hbnkgcmVnaW9ucyBbIDEsIDJdLiBUcmFkaXRpb25hbCBpcnJpZ2F0aW9uIHByYWN0aWNlcywgb2Z0ZW4gaW5lZmZpY2llbnQgYW5kIHJlc291cmNlLWludGVuc2l2ZSwgZXhhY2VyYmF0ZSB0aGVzZSBpc3N1ZXMsIGxlYWRpbmcgdG8gd2F0ZXIgd2FzdGFnZSwgZW5lcmd5IG92ZXJ1c2UsIGFuZCByZWR1Y2VkIGNyb3AgeWllbGRzLiJ9XQ%3D%3D.

### Recycled Wastewater for Irrigation

Recycled wastewater, encompassing treated municipal and industrial effluents, serves as a viable alternative to freshwater for irrigation, recycling nutrients and reducing discharge into natural water bodies while addressing agriculture's substantial 70% share of global freshwater withdrawals (Daraz et al., 2025)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlVtYXIiLCJmYW1pbHkiOiJEYXJheiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiLFoHRlZmFuIiwiZmFtaWx5IjoiQm9qbmVjIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllvdW5hcyIsImZhbWlseSI6IktoYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMDc3LTA0NzIiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzOTAvYWdyaWN1bHR1cmUxNTA1MDU1NCIsIm9wZW5hbGV4IjoiVzQ0MDgyMTk4MTEiLCJleGEiOiJodHRwczovL3d3dy5tZHBpLmNvbS8yMDc3LTA0NzIvMTUvNS81NTQifSwidGl0bGUiOiJFbmVyZ3ktRWZmaWNpZW50IFNtYXJ0IElycmlnYXRpb24gVGVjaG5vbG9naWVzOiBBIFBhdGh3YXkgdG8gV2F0ZXIgYW5kIEVuZXJneSBTdXN0YWluYWJpbGl0eSBpbiBBZ3JpY3VsdHVyZSIsImFic3RyYWN0IjoiVGhlIGFncmljdWx0dXJhbCBzZWN0b3IgZmFjZXMgY2hhbGxlbmdlcyBzdWNoIGFzIHdhdGVyIHNjYXJjaXR5LCBlbmVyZ3kgaW5lZmZpY2llbmN5LCBhbmQgZGVjbGluaW5nIHByb2R1Y3Rpdml0eSwgcGFydGljdWxhcmx5IGluIGFyaWQgcmVnaW9ucy4gVHJhZGl0aW9uYWwgaXJyaWdhdGlvbiBtZXRob2RzIGNvbnRyaWJ1dGUgdG8gcmVzb3VyY2UgZGVwbGV0aW9uIGFuZCBlbnZpcm9ubWVudGFsIGltcGFjdHMuIFNvbGFyLXBvd2VyZWQgc21hcnQgaXJyaWdhdGlvbiBzeXN0ZW1zIGludGVncmF0ZSBwcmVjaXNpb24gaXJyaWdhdGlvbiB3aXRoIHJlbmV3YWJsZSBlbmVyZ3ksIGltcHJvdmluZyB3YXRlciB1c2UgYW5kIHByb2R1Y3Rpdml0eS4gSW4gUGFraXN0YW4sIHdoZXJlIGFncmljdWx0dXJlIGNvbnRyaWJ1dGVzIDE5JSBvZiBncm9zcyBkb21lc3RpYyBwcm9kdWN0IGFuZCBlbXBsb3lzIDQwJSBvZiB0aGUgd29ya2ZvcmNlLCB0aGVzZSBjaGFsbGVuZ2VzIGFyZSBzZXZlcmUsIGVzcGVjaWFsbHkgaW4gd2F0ZXItc2NhcmNlIGFyZWFzIGxpa2UgdGhlIENob2xpc3RhbiBEZXNlcnQuIFRoaXMgc3R1ZHkgZXhhbWluZXMgdGhlIGltcGFjdCBvZiBzb2xhci1wb3dlcmVkIHNtYXJ0IGlycmlnYXRpb24gb24gYWdyaWN1bHR1cmFsIHByb2R1Y3Rpdml0eSwgd2F0ZXIgY29uc2VydmF0aW9uLCBhbmQgZW5lcmd5IGVmZmljaWVuY3kgaW4gdGhlIENob2xpc3RhbiBEZXNlcnQuIFVzaW5nIGEgcXVhbnRpdGF0aXZlIGNyb3NzLXNlY3Rpb25hbCBkZXNpZ24sIGRhdGEgd2VyZSBjb2xsZWN0ZWQgZnJvbSAzODQgZmFybWVycyB2aWEgc3RydWN0dXJlZCBxdWVzdGlvbm5haXJlcy4gU3RhdGlzdGljYWwgYW5hbHlzZXMsIGluY2x1ZGluZyBtdWx0aXBsZSBsaW5lYXIgcmVncmVzc2lvbiwgcGFpcmVkIHNhbXBsZSB0LXRlc3RzLCBhbmQgU3RydWN0dXJhbCBFcXVhdGlvbiBNb2RlbGluZyAoU0VNKSwgd2VyZSBjb25kdWN0ZWQuIEZpbmRpbmdzIHNob3cgc2lnbmlmaWNhbnQgaW1wcm92ZW1lbnRzIGluIGNyb3AgeWllbGQgKGZyb20gMy4wIHRvIDQuOCB0b25zL2hlY3RhcmUpIGFuZCByZWR1Y3Rpb25zIGluIHdhdGVyIGFuZCBlbmVyZ3kgY29uc3VtcHRpb24uIFJlZ3Jlc3Npb24gYW5hbHlzaXMgaGlnaGxpZ2h0ZWQgc3Ryb25nIHBvc2l0aXZlIGVmZmVjdHMgb24geWllbGQgYW5kIGVmZmljaWVuY3ksIHdoaWxlIFNFTSBjb25maXJtZWQgcmVkdWNlZCBlbnZpcm9ubWVudGFsIGltcGFjdCBhbmQgb3BlcmF0aW9uYWwgY29zdHMuIFRoZSBzdHVkeSBjb25jbHVkZXMgdGhhdCBzb2xhci1wb3dlcmVkIGlycmlnYXRpb24gZW5oYW5jZXMgcHJvZHVjdGl2aXR5LCBjb25zZXJ2ZXMgcmVzb3VyY2VzLCBhbmQgcHJvbW90ZXMgc3VzdGFpbmFiaWxpdHkuIFBvbGljeW1ha2VycyBzaG91bGQgcHJvdmlkZSBmaW5hbmNpYWwgaW5jZW50aXZlcywgaW52ZXN0IGluIHJlbmV3YWJsZSBpbmZyYXN0cnVjdHVyZSwgYW5kIGltcGxlbWVudCB0cmFpbmluZyBwcm9ncmFtcyB0byBzdXBwb3J0IGFkb3B0aW9uLiBDb2xsYWJvcmF0aXZlIGVmZm9ydHMgYXJlIGVzc2VudGlhbCBmb3Igc3VzdGFpbmFibGUgYWdyaWN1bHR1cmUgaW4gYXJpZCByZWdpb25zLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJpc3N1ZWREYXRlIjoiMjAyNS0wMy0wNSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkFncmljdWx0dXJlIiwiaXNzdWUiOjUsInZvbHVtZSI6MTUsImltcGFjdEZhY3RvciI6IjUuMjYifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjo1NTQsImxhc3RQYWdlIjo1NTR9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiTXVsdGlkaXNjaXBsaW5hcnkgRGlnaXRhbCBQdWJsaXNoaW5nIEluc3RpdHV0ZSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NDgsImNpdGVkQnlDb3VudCI6Mn19LCJxdW90ZSI6IjEuIEludHJvZHVjdGlvbiBUaGUgYWdyaWN1bHR1cmFsIHNlY3RvciwgYSBjb3JuZXJzdG9uZSBvZiBnbG9iYWwgZm9vZCBzZWN1cml0eSBhbmQgZWNvbm9taWMgZGV2ZWxvcG1lbnQsIGZhY2VzIG1vdW50aW5nIGNoYWxsZW5nZXMgaW4gdGhlIDIxc3QgY2VudHVyeS4gQ2xpbWF0ZSBjaGFuZ2UsIHJhcGlkIHBvcHVsYXRpb24gZ3Jvd3RoLCBhbmQgZHdpbmRsaW5nIG5hdHVyYWwgcmVzb3VyY2VzIGhhdmUgaW50ZW5zaWZpZWQgdGhlIG5lZWQgZm9yIHN1c3RhaW5hYmxlIGFncmljdWx0dXJhbCBwcmFjdGljZXMuIEFtb25nIHRoZXNlIGNoYWxsZW5nZXMsIHdhdGVyIHNjYXJjaXR5IGFuZCBlbmVyZ3kgaW5lZmZpY2llbmN5IHN0YW5kIG91dCBhcyBjcml0aWNhbCBiYXJyaWVycyB0byBwcm9kdWN0aXZpdHkgYW5kIHN1c3RhaW5hYmlsaXR5LiBHbG9iYWxseSwgYWdyaWN1bHR1cmUgYWNjb3VudHMgZm9yIGFwcHJveGltYXRlbHkgNzAlIG9mIGZyZXNod2F0ZXIgd2l0aGRyYXdhbHMgYW5kIGlzIG9uZSBvZiB0aGUgbGFyZ2VzdCBjb25zdW1lcnMgb2YgZW5lcmd5IGluIG1hbnkgcmVnaW9ucyBbIDEsIDJdLiBUcmFkaXRpb25hbCBpcnJpZ2F0aW9uIHByYWN0aWNlcywgb2Z0ZW4gaW5lZmZpY2llbnQgYW5kIHJlc291cmNlLWludGVuc2l2ZSwgZXhhY2VyYmF0ZSB0aGVzZSBpc3N1ZXMsIGxlYWRpbmcgdG8gd2F0ZXIgd2FzdGFnZSwgZW5lcmd5IG92ZXJ1c2UsIGFuZCByZWR1Y2VkIGNyb3AgeWllbGRzLiJ9XQ%3D%3D. This approach enhances water security, particularly in water-scarce regions, by transforming waste into a resource for crop production.

Principles of Operation

Advanced treatment processes, including biological nutrient removal, membrane filtration, and disinfection (e.g., UV or chlorination), ensure effluent quality meets agricultural reuse standards, with distribution via drip or sprinkler systems to minimize contamination risks.

Advantages and Efficiency Gains

- Nutrient recycling: Supplies essential NPK, cutting fertilizer needs by 20-30%;

- Water volume augmentation: Provides consistent supply independent of rainfall (Kushwaha et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjMwLCJzdWJ0eXBlIjp7fSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTWFuaXNoIiwiZmFtaWx5IjoiS3VzaHdhaGEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2hhbmthciIsImZhbWlseSI6IlNpbmdoIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlZpamF5IEthbnQiLCJmYW1pbHkiOiJTaW5naCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTaGFzaGFuayIsImZhbWlseSI6IkR3aXZlZGkifX1dLCJpZHMiOnsiaXNzbiI6WyIyNDU2LTE4NzgiXSwiaXNibiI6W10sImRvaSI6IjEwLjIyMTYxL2lqZWFiLjkyLjI3Iiwib3BlbmFsZXgiOiJXNDM5NjU2MzU4NCJ9LCJ0aXRsZSI6IlByZWNpc2lvbiBGYXJtaW5nOiBBIFJldmlldyBvZiBNZXRob2RzLCBUZWNobm9sb2dpZXMsIGFuZCBGdXR1cmUgUHJvc3BlY3RzIiwiYWJzdHJhY3QiOiJQcmVjaXNpb24gZmFybWluZywgb2Z0ZW4gcmVmZXJyZWQgdG8gYXMgcHJlY2lzaW9uIGFncmljdWx0dXJlLCBpcyBhIGZhcm1pbmcgbWFuYWdlbWVudCBjb25jZXB0IHRoYXQgc2Vla3MgdG8gbWF4aW1pemUgYWdyaWN1bHR1cmFsIG9wZXJhdGlvbnMgdmlhIHRoZSB1c2Ugb2YgdGVjaG5vbG9neSBpbiBvcmRlciB0byBhY2hpZXZlIGdyZWF0ZXIgbGV2ZWxzIG9mIGVmZmljaWVuY3ksIHByb2R1Y3Rpb24sIGFuZCBjb25zZXJ2YXRpb24gb2YgcmVzb3VyY2VzLiBUaGUgcHVycG9zZSBvZiB0aGlzIHN1cnZleSBzdHVkeSBpcyB0byBvZmZlciBhbiBvdmVydmlldyBvZiBwcmVjaXNpb24gZmFybWluZywgaW5jbHVkaW5nIHRvcGljcyBzdWNoIGFzIGl0cyBmdW5kYW1lbnRhbHMsIG1ldGhvZG9sb2dpZXMsIHRlY2hub2xvZ3ksIGFuZCBhcHBsaWNhdGlvbnMuIEl0IGludmVzdGlnYXRlcyB0aGUgcm9sZSB0aGF0IGEgdmFyaWV0eSBvZiB0ZWNobm9sb2dpZXMsIGluY2x1ZGluZyBnbG9iYWwgbmF2aWdhdGlvbiBzYXRlbGxpdGUgc3lzdGVtcyAoR05TUyksIGdlb2dyYXBoaWMgaW5mb3JtYXRpb24gc3lzdGVtcyAoR0lTKSwgcmVtb3RlIHNlbnNpbmcsIGFuZCBzZW5zb3IgdGVjaG5vbG9neSwgcGxheSBpbiBtYWtpbmcgaXQgcG9zc2libGUgdG8gaW1wbGVtZW50IHByZWNpc2lvbiBhZ3JpY3VsdHVyYWwgbWV0aG9kcy4gSW4gYWRkaXRpb24sIGl0IGFkZHJlc3NlcyB0aGUgYWR2YW50YWdlcywgZGlzYWR2YW50YWdlcywgYW5kIHBvdGVudGlhbCBmdXR1cmUgYXBwbGljYXRpb25zIG9mIHByZWNpc2lvbiBmYXJtaW5nIGluIHRoZSBjb250ZXh0IG9mIHNvbHZpbmcgaXNzdWVzIHJlbGF0ZWQgdG8gdGhlIHN1c3RhaW5hYmlsaXR5IG9mIGFncmljdWx0dXJlIGFuZCB0aGUgc2VjdXJpdHkgb2YgZm9vZCBzdXBwbGllcyBvbiBhIHdvcmxkd2lkZSBzY2FsZS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjIxNjEvaWplYWIuOTIuMjciLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vaWplYWIuY29tL3VwbG9hZF9kb2N1bWVudC9pc3N1ZV9maWxlcy8yN0lKRUFCLTEwNDIwMjQ4NC1QcmVjaXNpb25GYXJtaW5nLnBkZiIsImlzc3VlZERhdGUiOiIyMDI0LTAxLTAxIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiSW50ZXJuYXRpb25hbCBKb3VybmFsIG9mIEVudmlyb25tZW50IEFncmljdWx0dXJlIGFuZCBCaW90ZWNobm9sb2d5IiwiaXNzdWUiOjIsInZvbHVtZSI6OSwiaW1wYWN0RmFjdG9yIjoiMC42NSJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjI0MiwibGFzdFBhZ2UiOjI1M319LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJJbmZvZ2FpbiBQdWJsaWNhdGlvbiJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NCwiY2l0ZWRCeUNvdW50Ijo4fX0sInF1b3RlIjoiSUlJLiBCRU5FRklUUyBPRiBQUkVDSVNJT04gSVJSSUdBVElPTiAxLiBJbXByb3ZlZCBXYXRlciBVc2UgRWZmaWNpZW5jeTogUHJlY2lzaW9uIGlycmlnYXRpb24gb3B0aW1pemVzIHdhdGVyIHVzZSBlZmZpY2llbmN5IGJ5IGRlbGl2ZXJpbmcgd2F0ZXIgZGlyZWN0bHkgdG8gdGhlIHJvb3Qgem9uZSBvZiBjcm9wcywgbWluaW1pemluZyBsb3NzZXMgIGR1ZSB0byBldmFwb3JhdGlvbiwgcnVub2ZmLCBhbmQgZGVlcCBwZXJjb2xhdGlvbi4gQnkgIG1hdGNoaW5nIHdhdGVyIGFwcGxpY2F0aW9uIHJhdGVzIHRvIGNyb3Agd2F0ZXIgIHJlcXVpcmVtZW50cywgcHJlY2lzaW9uIGlycmlnYXRpb24gc3lzdGVtcyBlbnN1cmUgdGhhdCAgd2F0ZXIgaXMgdXNlZCBtb3JlIGVmZmljaWVudGx5LCBtYXhpbWl6aW5nIGNyb3AgeWllbGRzICBwZXIgdW5pdCBvZiB3YXRlciBhcHBsaWVkLiAyLiBFbmhhbmNlZCBDcm9wIFBlcmZvcm1hbmNlOiBCeSBwcm92aWRpbmcgY3JvcHMgd2l0aCBvcHRpbWFsIG1vaXN0dXJlIGxldmVscywgcHJlY2lzaW9uIGlycmlnYXRpb24gc3lzdGVtcyBwcm9tb3RlIGhlYWx0aHkgcm9vdCBkZXZlbG9wbWVudCwgYmFsYW5jZWQgZ3Jvd3RoLCAgYW5kIGltcHJvdmVkIGNyb3Agdmlnb3IuIENvbnNpc3RlbnQgc29pbCBtb2lzdHVyZSAgbGV2ZWxzIGhlbHAgcmVkdWNlIHdhdGVyIHN0cmVzcywgaW5jcmVhc2UgbnV0cmllbnQgIHVwdGFrZSwgYW5kIGVuaGFuY2UgY3JvcCByZXNpbGllbmNlIHRvIGVudmlyb25tZW50YWwgIHN0cmVzc2VzIHN1Y2ggYXMgZHJvdWdodCBhbmQgaGVhdC4gMy4gQ29zdCBTYXZpbmdzOiBQcmVjaXNpb24gaXJyaWdhdGlvbiBzeXN0ZW1zIGhlbHAgZmFybWVycyByZWR1Y2Ugd2F0ZXIsIGVuZXJneSwgYW5kIGxhYm9yIGNvc3RzIGFzc29jaWF0ZWQgd2l0aCBpcnJpZ2F0aW9uLiBCeSBvcHRpbWl6aW5nIHdhdGVyIHVzZSBlZmZpY2llbmN5IGFuZCAgbWluaW1pemluZyBpbnB1dCB3YXN0YWdlLCBwcmVjaXNpb24gaXJyaWdhdGlvbiBzeXN0ZW1zICBjYW4gbGVhZCB0byBzaWduaWZpY2FudCBjb3N0IHNhdmluZ3Mgb3ZlciB0aW1lLCAgaW1wcm92aW5nIHRoZSBwcm9maXRhYmlsaXR5IG9mIGFncmljdWx0dXJhbCBvcGVyYXRpb25zLiA0LiBFbnZpcm9ubWVudGFsIFN1c3RhaW5hYmlsaXR5OiAgUHJlY2lzaW9uICBpcnJpZ2F0aW9uIGNvbnRyaWJ1dGVzIHRvICBlbnZpcm9ubWVudGFsICBzdXN0YWluYWJpbGl0eSAgYnkgY29uc2VydmluZyB3YXRlciAgcmVzb3VyY2VzLCAgbWluaW1pemluZyAgd2F0ZXIgcG9sbHV0aW9uLCBhbmQgcmVkdWNpbmcgZ3JlZW5ob3VzZSBnYXMgZW1pc3Npb25zLiBCeSBhZG9wdGluZyBwcmVjaXNpb24gaXJyaWdhdGlvbiBwcmFjdGljZXMsIGZhcm1lcnMgY2FuICBtaW5pbWl6ZSB0aGVpciBlbnZpcm9ubWVudGFsIGZvb3RwcmludCBhbmQgY29udHJpYnV0ZSB0byB0aGUgIGxvbmctdGVybSAgc3VzdGFpbmFiaWxpdHkgIG9mICBhZ3JpY3VsdHVyYWwgcHJvZHVjdGlvbiBzeXN0ZW1zLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjQ0Ljg2NTY0MjU0NzYwNzQyLCJsZWZ0Ijo1My40MDgyNTI3MTYwNjQ0NSwid2lkdGgiOjM4LjI2MTc2ODM0MTA2NDQ1LCJoZWlnaHQiOjEuNjY5MjI3MzYxNjc5MDc3MX19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjQ2Ljg5NzkyMjUxNTg2OTE0LCJsZWZ0Ijo1Mi40MjA1NDM2NzA2NTQzLCJ3aWR0aCI6NDAuMjQwNzM3OTE1MDM5MDYsImhlaWdodCI6MS42MjU0NTQ3ODM0Mzk2MzYyfX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NDguNDkxNzkwNzcxNDg0Mzc1LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3Ljk4MDI3ODAxNTEzNjcyLCJoZWlnaHQiOjExLjAxMzcxMzgzNjY2OTkyMn19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjU5LjkzOTU0MDg2MzAzNzExLCJsZWZ0Ijo1Mi40MjA1NDM2NzA2NTQzLCJ3aWR0aCI6NDAuMjMwMjI0NjA5Mzc1LCJoZWlnaHQiOjEuNjI1NDU0NzgzNDM5NjM2Mn19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjYxLjUzMzQxNjc0ODA0Njg3NSwibGVmdCI6NTQuNjk4MzEwODUyMDUwNzgsIndpZHRoIjozNy45NjMxMjMzMjE1MzMyLCJoZWlnaHQiOjkuNDM1MTcwMTczNjQ1MDJ9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo3MS40MTY4Nzc3NDY1ODIwMywibGVmdCI6NTIuNDIwNTQzNjcwNjU0Mywid2lkdGgiOjQwLjIxMzMxMDI0MTY5OTIyLCJoZWlnaHQiOjEuNjI1NDU4MzU5NzE4MzIyOH19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjczLjAxMDc1NzQ0NjI4OTA2LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3Ljk0NTc4NTUyMjQ2MDk0LCJoZWlnaHQiOjcuODYzNzY0Mjg2MDQxMjZ9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo4MS4zMDg1NTU2MDMwMjczNCwibGVmdCI6NTIuNDIwNTQzNjcwNjU0Mywid2lkdGgiOjQwLjI0MDM3NTUxODc5ODgzLCJoZWlnaHQiOjEuNjI1NDU4MzU5NzE4MzIyOH19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjgyLjkxOTA1OTc1MzQxNzk3LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3LjkzNjQxNjYyNTk3NjU2LCJoZWlnaHQiOjEuNTc4MTMzNTgzMDY4ODQ3N319LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjg0LjQ4NjkwMDMyOTU4OTg0LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3LjkzNDc1NzIzMjY2NjAxNiwiaGVpZ2h0IjoxLjU3ODEzMzU4MzA2ODg0Nzd9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo4Ni4wNTQ3NDg1MzUxNTYyNSwibGVmdCI6NTQuNjk4MzEwODUyMDUwNzgsIndpZHRoIjozNy45Nzg0MzE3MDE2NjAxNTYsImhlaWdodCI6NC43MTM4MTkwMjY5NDcwMjE1fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6OC40MjAyOTI4NTQzMDkwODIsImxlZnQiOjEwLjAyMjE3MjkyNzg1NjQ0NSwid2lkdGgiOjM3Ljk0NjgzMDc0OTUxMTcyLCJoZWlnaHQiOjEuNTc4MTM1MjUxOTk4OTAxNH19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjkuOTg4MTMyNDc2ODA2NjQsImxlZnQiOjEwLjAyMjE3MjkyNzg1NjQ0NSwid2lkdGgiOjEzLjg2MjQ2MTA5MDA4Nzg5LCJoZWlnaHQiOjEuNTc4MTM1MjUxOTk4OTAxNH19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjI0OSJ9fV0%3D;

- Pollution mitigation: Prevents eutrophication in waterways.

Applications and Implementation

Common in water-stressed areas like Israel (90% reuse), Australia, and parts of California for non-food crops and orchards; integrated with precision systems for safe application (Jumintono et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikp1bWludG9ubyIsImZhbWlseSI6Ikp1bWludG9ubyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJOdXJuYXdhdHkiLCJmYW1pbHkiOiJOdXJuYXdhdHkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTnVncm9obyBUcmlzbnUiLCJmYW1pbHkiOiJCcmF0YSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJEaW5pIiwiZmFtaWx5IjoiUmFobWlhdGkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSGFkaSIsImZhbWlseSI6IlN1c2lsbyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBLiBBLiIsImZhbWlseSI6Ikh1c3NlbiJ9fV0sImlkcyI6eyJpc3NuIjpbIjE0MjktNzQyNiIsIjIwODMtNDUzNSJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMjQ0MjUvandsZC4yMDI0LjE1MTc5OCIsIm9wZW5hbGV4IjoiVzQ0MDU0NzcxMDAifSwidGl0bGUiOiJDb21wYXJhdGl2ZSBhbmFseXNpcyBvZiBwcmVjaXNpb24sIGRlZmljaXQsIGFuZCBkcmlwIGlycmlnYXRpb24gZm9yIHdhdGVyIGVmZmljaWVuY3kgaW4gSXJhcSIsImFic3RyYWN0IjoiU3VzdGFpbmFibGUgaXJyaWdhdGlvbiBwcmFjdGljZXMgYXJlIGNyaXRpY2FsIGZvciBhZGRyZXNzaW5nIHRoZSBjaGFsbGVuZ2VzIG9mIHdhdGVyIHNjYXJjaXR5IGFuZCBmb29kIGluc2VjdXJpdHkgaW4gdGhlIGZhY2Ugb2YgY2xpbWF0ZSBjaGFuZ2UgYW5kIGdyb3dpbmcgZ2xvYmFsIGZvb2QgZGVtYW5kLiBUaGlzIHN0dWR5IGludmVzdGlnYXRlZCB0aGUgcG90ZW50aWFsIG9mIHByZWNpc2lvbiBpcnJpZ2F0aW9uIChQSSksIGRlZmljaXQgaXJyaWdhdGlvbiAoREkpLCBhbmQgZHJpcCBpcnJpZ2F0aW9uIChEUkkpIHRlY2huaXF1ZXMgdG8gZW5oYW5jZSBjcm9wIHByb2R1Y3Rpdml0eSBhbmQgd2F0ZXIgdXNlIGVmZmljaWVuY3kgKFdVRSkgY29tcGFyZWQgdG8gY29udmVudGlvbmFsIGZsb29kIGlycmlnYXRpb24gKENGSSkgaW4gdGhyZWUgZGlzdGluY3QgYWdyb2Vjb2xvZ2ljYWwgem9uZXMgb2YgSXJhcS4gRmllbGQgZXhwZXJpbWVudHMgd2VyZSBjb25kdWN0ZWQgdXNpbmcgYSByYW5kb21pc2VkIGNvbXBsZXRlIGJsb2NrIGRlc2lnbiB3aXRoIHdoZWF0LCBtYWl6ZSwgYW5kIHJpY2UgY3JvcHMuIFRoZSByZXN1bHRzIGRlbW9uc3RyYXRlZCB0aGF0IFBJIHNpZ25pZmljYW50bHkgaW5jcmVhc2VkIGNyb3AgZ3Jvd3RoIHBhcmFtZXRlcnMsIGdyYWluIHlpZWxkLCBhbmQgV1VFIGFjcm9zcyBhbGwgem9uZXMsIHdpdGggeWllbGQgaW1wcm92ZW1lbnRzIG9mIDMz4oCTMzglIGFuZCBXVUUgaW5jcmVhc2VzIG9mIDQ24oCTNTElIGluIGNvbnRyYXN0IHdpdGggQ0ZJLiBUaGUgREkgYW5kIERSSSB0cmVhdG1lbnRzIGFsc28gb3V0cGVyZm9ybWVkIENGSSwgYWxiZWl0IHRvIGEgbGVzc2VyIGV4dGVudC4gUmVtb3RlIHNlbnNpbmctZGVyaXZlZCB2ZWdldGF0aW9uIGluZGljZXMgc3Ryb25nbHkgY29ycmVsYXRlZCB3aXRoIGNyb3AgZ3Jvd3RoIHBhcmFtZXRlcnMgYW5kIHlpZWxkLCB3aGlsZSBoeWRyb2xvZ2ljYWwgbW9kZWxsaW5nIHJldmVhbGVkIHJlZHVjZWQgZXZhcG90cmFuc3BpcmF0aW9uIGFuZCBzdXJmYWNlIHJ1bm9mZiB1bmRlciB0aGUgUEkgdHJlYXRtZW50LiBUaGUgc3VzdGFpbmFibGUgaXJyaWdhdGlvbiBwcmFjdGljZXMgcmVzdWx0ZWQgaW4gc3Vic3RhbnRpYWwgd2F0ZXIgc2F2aW5ncyBvZiAyMOKAkzMwJSBjb21wYXJlZCB0byBDRkkuIFRoZXNlIGZpbmRpbmdzIGhpZ2hsaWdodCB0aGUgaW1wb3J0YW5jZSBvZiBhZG9wdGluZyBlZmZpY2llbnQgaXJyaWdhdGlvbiB0ZWNobmlxdWVzLCBhbG9uZyB3aXRoIGEgaG9saXN0aWMgYXBwcm9hY2ggZW5jb21wYXNzaW5nIHRlY2hub2xvZ2ljYWwgaW5ub3ZhdGlvbnMsIGNhcGFjaXR5IGJ1aWxkaW5nLCBhbmQgc3Rha2Vob2xkZXIgZW5nYWdlbWVudCwgdG8gcHJvbW90ZSB3YXRlci0gZWZmaWNpZW50IGFncmljdWx0dXJlIGFuZCBlbnN1cmUgZm9vZCBzZWN1cml0eSBpbiB3YXRlci1zY2FyY2UgcmVnaW9ucy4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjQ0MjUvandsZC4yMDI0LjE1MTc5OCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjI0NDI1L2p3bGQuMjAyNC4xNTE3OTgiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMi0xNCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkpvdXJuYWwgb2YgV2F0ZXIgYW5kIExhbmQgRGV2ZWxvcG1lbnQiLCJpbXBhY3RGYWN0b3IiOiIxLjQ2In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTMwLCJsYXN0UGFnZSI6MTM2fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkRlIEdydXl0ZXIgT3BlbiJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MzIsImNpdGVkQnlDb3VudCI6MX19LCJxdW90ZSI6IkluIHRoaXMgY29udGV4dCwgdGhlIGFkb3B0aW9uIG9mICBzdXN0YWluYWJsZSBpcnJpZ2F0aW9uIHByYWN0aWNlcyBoYXMgZW1lcmdlZCBhcyBhIGNyaXRpY2FsIHN0cmF0ZWd5ICBmb3IgZW5oYW5jaW5nIGZvb2Qgc2VjdXJpdHkgd2hpbGUgcHJvbW90aW5nIHdhdGVyIGNvbnNlcnZhdGlvbiAgaW4gYWdyaWN1bHR1cmFsIGxhbmRzY2FwZXMuICBJcnJpZ2F0aW9uIHBsYXlzIGEgdml0YWwgcm9sZSBpbiBnbG9iYWwgZm9vZCBwcm9kdWN0aW9uLCB3aXRoICBpcnJpZ2F0ZWQgYWdyaWN1bHR1cmUgYWNjb3VudGluZyBmb3IgYXBwcm94aW1hdGVseSA0MCUgb2YgdGhlICBmb29kIHN1cHBseSAoV3JhY2hpZW4gZGUsIFNjaHVsdHogYW5kIEdvbGksIDIwMjE7IFNjaG1pdHQsIFJvc2EgIGFuZCBEYWlseSwgMjAyMikuIEhvd2V2ZXIsIHRyYWRpdGlvbmFsIGlycmlnYXRpb24gbWV0aG9kcywgc3VjaCBhcyAgZmxvb2QgYW5kIGZ1cnJvdyBpcnJpZ2F0aW9uLCBhcmUgb2Z0ZW4gaW5lZmZpY2llbnQsIGxlYWRpbmcgdG8gIHN1YnN0YW50aWFsIHdhdGVyIGxvc3NlcyB0aHJvdWdoIGV2YXBvcmF0aW9uLCBydW5vZmYsIGFuZCBkZWVwICBwZXJjb2xhdGlvbiAoQW1ib21zYSwgMjAyMCkuIE1vcmVvdmVyLCB0aGUgZXhjZXNzaXZlIGFwcGxpY2EtdGlvbiBvZiB3YXRlciBjYW4gcmVzdWx0IGluIHNvaWwgc2FsaW5pc2F0aW9uLCBudXRyaWVudCBsZWFjaGluZywgYW5kICBncm91bmR3YXRlciBkZXBsZXRpb24sIGZ1cnRoZXIgY29tcHJvbWlzaW5nIHRoZSBsb25nLXRlcm0gIHN1c3RhaW5hYmlsaXR5IG9mIGFncmljdWx0dXJhbCBzeXN0ZW1zIChZaW4gZXQgYWwuLCAyMDIyKS4gVG8gYWRkcmVzcyAgdGhlc2UgY2hhbGxlbmdlcywgcmVzZWFyY2hlcnMgYW5kIHByYWN0aXRpb25lcnMgaGF2ZSBiZWVuIGV4cGxvci1pbmcgaW5ub3ZhdGl2ZSBpcnJpZ2F0aW9uIHRlY2huaXF1ZXMgYW5kIHdhdGVyIG1hbmFnZW1lbnQgIHN0cmF0ZWdpZXMgdGhhdCBjYW4gb3B0aW1pc2Ugd2F0ZXIgdXNlIGVmZmljaWVuY3kgKFdVRSkgd2hpbGUgIG1haW50YWluaW5nIG9yIGVuaGFuY2luZyBjcm9wIHByb2R1Y3Rpdml0eS4gIFByZWNpc2lvbiBpcnJpZ2F0aW9uIChQSSkgb2ZmZXJzIGEgcHJvbWlzaW5nIHNvbHV0aW9uLCAgaW52b2x2aW5nIHRoZSB0YXJnZXRlZCBhcHBsaWNhdGlvbiBvZiB3YXRlciBzcGVjaWZpY2FsbHkgdG8gdGhlICBuZWVkcyBvZiBpbmRpdmlkdWFsIHBsYW50cyBvciBjcm9wIHpvbmVzIChaaW5rZXJuYWdlbCBldCBhbC4sIDIwMjApLiAgIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyMi44ODY3MzU5MTYxMzc2OTUsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzQ2MjIxOTIzODI4MTI1LCJoZWlnaHQiOjEuNDUwNTQxMDE5NDM5Njk3M319LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI0LjM1MTEyMzgwOTgxNDQ1MywibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTI3MTYwNjQ0NSwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI1LjcyNDk4NzAzMDAyOTI5NywibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTY1MzA3NjE3MiwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI3LjA5ODg1MDI1MDI0NDE0LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjE2LjQxODE2MTM5MjIxMTkxNCwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI4LjQ3MjcwOTY1NTc2MTcyLCJsZWZ0IjoxMi4zODA4ODAzNTU4MzQ5NjEsIndpZHRoIjozNy4wMTIxODQxNDMwNjY0MDYsImhlaWdodCI6MS4zNjAwMTQ3OTYyNTcwMTl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyOS44NDY1NzI4NzU5NzY1NjIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQwNjQ4NjUxMTIzMDUsImhlaWdodCI6MS4zNjAwMTY3MDM2MDU2NTE5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzEuMjE0MDM2OTQxNTI4MzIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQ0ODU2MjYyMjA3MDMsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzIuNTg3OTAyMDY5MDkxOCwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDYwNTc4OTE4NDU3LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMzLjk2MTc2MTQ3NDYwOTM3NSwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDA0ODQ2MTkxNDA2MjUsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzUuMzM1NjI0Njk0ODI0MjIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzM5NzQ0NTY3ODcxMDk0LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjM2LjcwOTQ4NzkxNTAzOTA2LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjAwMTU5MDcyODc1OTc2NiwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC4wODMzNDczMjA1NTY2NCwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTY1MzA3NjE3MiwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOS40NTcyMTA1NDA3NzE0ODQsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQzMDQ4MDk1NzAzMTI1LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQwLjc0MDU0NzE4MDE3NTc4LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjM0NTQ1NTE2OTY3NzczNCwiaGVpZ2h0IjoxLjQ1MDU0MjgwNzU3OTA0MDV9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Mi4yMDQ5MzMxNjY1MDM5MDYsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMDAzMjUwMTIyMDcwMzEsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDMuNTc4Nzk2Mzg2NzE4NzUsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzM3OTIxMTQyNTc4MTI1LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQ0Ljg2MjEzMzAyNjEyMzA1LCJsZWZ0Ijo5LjA0NzYwMTY5OTgyOTEwMiwid2lkdGgiOjQwLjM0NTMxNDAyNTg3ODkwNiwiaGVpZ2h0IjoxLjQ1MDU0MjgwNzU3OTA0MDV9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ni4zMjY1MTkwMTI0NTExNywibGVmdCI6OS4wNDc2MDE2OTk4MjkxMDIsIndpZHRoIjoyNy45Njc0NDcyODA4ODM3OSwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ny43MDAzODIyMzI2NjYwMTYsImxlZnQiOjEyLjM4MDg3ODQ0ODQ4NjMyOCwid2lkdGgiOjM3LjAzNDc4NjIyNDM2NTIzNCwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0OS4wNzQyNDU0NTI4ODA4NiwibGVmdCI6OS4wNDc2MDE2OTk4MjkxMDIsIndpZHRoIjo0MC4zNDA2NDQ4MzY0MjU3OCwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1MC4zNTc1ODIwOTIyODUxNTYsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzQ0ODUyNDQ3NTA5NzY2LCJoZWlnaHQiOjEuNDUwNTQyODA3NTc5MDQwNX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEzMSJ9fV0%3D.

Limitations and Challenges

Pathogen risks, heavy metal accumulation, public perception barriers, and high treatment costs; requires strict monitoring and regulatory compliance (Kushwaha et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjMwLCJzdWJ0eXBlIjp7fSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTWFuaXNoIiwiZmFtaWx5IjoiS3VzaHdhaGEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2hhbmthciIsImZhbWlseSI6IlNpbmdoIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlZpamF5IEthbnQiLCJmYW1pbHkiOiJTaW5naCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTaGFzaGFuayIsImZhbWlseSI6IkR3aXZlZGkifX1dLCJpZHMiOnsiaXNzbiI6WyIyNDU2LTE4NzgiXSwiaXNibiI6W10sImRvaSI6IjEwLjIyMTYxL2lqZWFiLjkyLjI3Iiwib3BlbmFsZXgiOiJXNDM5NjU2MzU4NCJ9LCJ0aXRsZSI6IlByZWNpc2lvbiBGYXJtaW5nOiBBIFJldmlldyBvZiBNZXRob2RzLCBUZWNobm9sb2dpZXMsIGFuZCBGdXR1cmUgUHJvc3BlY3RzIiwiYWJzdHJhY3QiOiJQcmVjaXNpb24gZmFybWluZywgb2Z0ZW4gcmVmZXJyZWQgdG8gYXMgcHJlY2lzaW9uIGFncmljdWx0dXJlLCBpcyBhIGZhcm1pbmcgbWFuYWdlbWVudCBjb25jZXB0IHRoYXQgc2Vla3MgdG8gbWF4aW1pemUgYWdyaWN1bHR1cmFsIG9wZXJhdGlvbnMgdmlhIHRoZSB1c2Ugb2YgdGVjaG5vbG9neSBpbiBvcmRlciB0byBhY2hpZXZlIGdyZWF0ZXIgbGV2ZWxzIG9mIGVmZmljaWVuY3ksIHByb2R1Y3Rpb24sIGFuZCBjb25zZXJ2YXRpb24gb2YgcmVzb3VyY2VzLiBUaGUgcHVycG9zZSBvZiB0aGlzIHN1cnZleSBzdHVkeSBpcyB0byBvZmZlciBhbiBvdmVydmlldyBvZiBwcmVjaXNpb24gZmFybWluZywgaW5jbHVkaW5nIHRvcGljcyBzdWNoIGFzIGl0cyBmdW5kYW1lbnRhbHMsIG1ldGhvZG9sb2dpZXMsIHRlY2hub2xvZ3ksIGFuZCBhcHBsaWNhdGlvbnMuIEl0IGludmVzdGlnYXRlcyB0aGUgcm9sZSB0aGF0IGEgdmFyaWV0eSBvZiB0ZWNobm9sb2dpZXMsIGluY2x1ZGluZyBnbG9iYWwgbmF2aWdhdGlvbiBzYXRlbGxpdGUgc3lzdGVtcyAoR05TUyksIGdlb2dyYXBoaWMgaW5mb3JtYXRpb24gc3lzdGVtcyAoR0lTKSwgcmVtb3RlIHNlbnNpbmcsIGFuZCBzZW5zb3IgdGVjaG5vbG9neSwgcGxheSBpbiBtYWtpbmcgaXQgcG9zc2libGUgdG8gaW1wbGVtZW50IHByZWNpc2lvbiBhZ3JpY3VsdHVyYWwgbWV0aG9kcy4gSW4gYWRkaXRpb24sIGl0IGFkZHJlc3NlcyB0aGUgYWR2YW50YWdlcywgZGlzYWR2YW50YWdlcywgYW5kIHBvdGVudGlhbCBmdXR1cmUgYXBwbGljYXRpb25zIG9mIHByZWNpc2lvbiBmYXJtaW5nIGluIHRoZSBjb250ZXh0IG9mIHNvbHZpbmcgaXNzdWVzIHJlbGF0ZWQgdG8gdGhlIHN1c3RhaW5hYmlsaXR5IG9mIGFncmljdWx0dXJlIGFuZCB0aGUgc2VjdXJpdHkgb2YgZm9vZCBzdXBwbGllcyBvbiBhIHdvcmxkd2lkZSBzY2FsZS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjIxNjEvaWplYWIuOTIuMjciLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vaWplYWIuY29tL3VwbG9hZF9kb2N1bWVudC9pc3N1ZV9maWxlcy8yN0lKRUFCLTEwNDIwMjQ4NC1QcmVjaXNpb25GYXJtaW5nLnBkZiIsImlzc3VlZERhdGUiOiIyMDI0LTAxLTAxIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiSW50ZXJuYXRpb25hbCBKb3VybmFsIG9mIEVudmlyb25tZW50IEFncmljdWx0dXJlIGFuZCBCaW90ZWNobm9sb2d5IiwiaXNzdWUiOjIsInZvbHVtZSI6OSwiaW1wYWN0RmFjdG9yIjoiMC42NSJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjI0MiwibGFzdFBhZ2UiOjI1M319LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJJbmZvZ2FpbiBQdWJsaWNhdGlvbiJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NCwiY2l0ZWRCeUNvdW50Ijo4fX0sInF1b3RlIjoiSUlJLiBCRU5FRklUUyBPRiBQUkVDSVNJT04gSVJSSUdBVElPTiAxLiBJbXByb3ZlZCBXYXRlciBVc2UgRWZmaWNpZW5jeTogUHJlY2lzaW9uIGlycmlnYXRpb24gb3B0aW1pemVzIHdhdGVyIHVzZSBlZmZpY2llbmN5IGJ5IGRlbGl2ZXJpbmcgd2F0ZXIgZGlyZWN0bHkgdG8gdGhlIHJvb3Qgem9uZSBvZiBjcm9wcywgbWluaW1pemluZyBsb3NzZXMgIGR1ZSB0byBldmFwb3JhdGlvbiwgcnVub2ZmLCBhbmQgZGVlcCBwZXJjb2xhdGlvbi4gQnkgIG1hdGNoaW5nIHdhdGVyIGFwcGxpY2F0aW9uIHJhdGVzIHRvIGNyb3Agd2F0ZXIgIHJlcXVpcmVtZW50cywgcHJlY2lzaW9uIGlycmlnYXRpb24gc3lzdGVtcyBlbnN1cmUgdGhhdCAgd2F0ZXIgaXMgdXNlZCBtb3JlIGVmZmljaWVudGx5LCBtYXhpbWl6aW5nIGNyb3AgeWllbGRzICBwZXIgdW5pdCBvZiB3YXRlciBhcHBsaWVkLiAyLiBFbmhhbmNlZCBDcm9wIFBlcmZvcm1hbmNlOiBCeSBwcm92aWRpbmcgY3JvcHMgd2l0aCBvcHRpbWFsIG1vaXN0dXJlIGxldmVscywgcHJlY2lzaW9uIGlycmlnYXRpb24gc3lzdGVtcyBwcm9tb3RlIGhlYWx0aHkgcm9vdCBkZXZlbG9wbWVudCwgYmFsYW5jZWQgZ3Jvd3RoLCAgYW5kIGltcHJvdmVkIGNyb3Agdmlnb3IuIENvbnNpc3RlbnQgc29pbCBtb2lzdHVyZSAgbGV2ZWxzIGhlbHAgcmVkdWNlIHdhdGVyIHN0cmVzcywgaW5jcmVhc2UgbnV0cmllbnQgIHVwdGFrZSwgYW5kIGVuaGFuY2UgY3JvcCByZXNpbGllbmNlIHRvIGVudmlyb25tZW50YWwgIHN0cmVzc2VzIHN1Y2ggYXMgZHJvdWdodCBhbmQgaGVhdC4gMy4gQ29zdCBTYXZpbmdzOiBQcmVjaXNpb24gaXJyaWdhdGlvbiBzeXN0ZW1zIGhlbHAgZmFybWVycyByZWR1Y2Ugd2F0ZXIsIGVuZXJneSwgYW5kIGxhYm9yIGNvc3RzIGFzc29jaWF0ZWQgd2l0aCBpcnJpZ2F0aW9uLiBCeSBvcHRpbWl6aW5nIHdhdGVyIHVzZSBlZmZpY2llbmN5IGFuZCAgbWluaW1pemluZyBpbnB1dCB3YXN0YWdlLCBwcmVjaXNpb24gaXJyaWdhdGlvbiBzeXN0ZW1zICBjYW4gbGVhZCB0byBzaWduaWZpY2FudCBjb3N0IHNhdmluZ3Mgb3ZlciB0aW1lLCAgaW1wcm92aW5nIHRoZSBwcm9maXRhYmlsaXR5IG9mIGFncmljdWx0dXJhbCBvcGVyYXRpb25zLiA0LiBFbnZpcm9ubWVudGFsIFN1c3RhaW5hYmlsaXR5OiAgUHJlY2lzaW9uICBpcnJpZ2F0aW9uIGNvbnRyaWJ1dGVzIHRvICBlbnZpcm9ubWVudGFsICBzdXN0YWluYWJpbGl0eSAgYnkgY29uc2VydmluZyB3YXRlciAgcmVzb3VyY2VzLCAgbWluaW1pemluZyAgd2F0ZXIgcG9sbHV0aW9uLCBhbmQgcmVkdWNpbmcgZ3JlZW5ob3VzZSBnYXMgZW1pc3Npb25zLiBCeSBhZG9wdGluZyBwcmVjaXNpb24gaXJyaWdhdGlvbiBwcmFjdGljZXMsIGZhcm1lcnMgY2FuICBtaW5pbWl6ZSB0aGVpciBlbnZpcm9ubWVudGFsIGZvb3RwcmludCBhbmQgY29udHJpYnV0ZSB0byB0aGUgIGxvbmctdGVybSAgc3VzdGFpbmFiaWxpdHkgIG9mICBhZ3JpY3VsdHVyYWwgcHJvZHVjdGlvbiBzeXN0ZW1zLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjQ0Ljg2NTY0MjU0NzYwNzQyLCJsZWZ0Ijo1My40MDgyNTI3MTYwNjQ0NSwid2lkdGgiOjM4LjI2MTc2ODM0MTA2NDQ1LCJoZWlnaHQiOjEuNjY5MjI3MzYxNjc5MDc3MX19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjQ2Ljg5NzkyMjUxNTg2OTE0LCJsZWZ0Ijo1Mi40MjA1NDM2NzA2NTQzLCJ3aWR0aCI6NDAuMjQwNzM3OTE1MDM5MDYsImhlaWdodCI6MS42MjU0NTQ3ODM0Mzk2MzYyfX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NDguNDkxNzkwNzcxNDg0Mzc1LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3Ljk4MDI3ODAxNTEzNjcyLCJoZWlnaHQiOjExLjAxMzcxMzgzNjY2OTkyMn19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjU5LjkzOTU0MDg2MzAzNzExLCJsZWZ0Ijo1Mi40MjA1NDM2NzA2NTQzLCJ3aWR0aCI6NDAuMjMwMjI0NjA5Mzc1LCJoZWlnaHQiOjEuNjI1NDU0NzgzNDM5NjM2Mn19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjYxLjUzMzQxNjc0ODA0Njg3NSwibGVmdCI6NTQuNjk4MzEwODUyMDUwNzgsIndpZHRoIjozNy45NjMxMjMzMjE1MzMyLCJoZWlnaHQiOjkuNDM1MTcwMTczNjQ1MDJ9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo3MS40MTY4Nzc3NDY1ODIwMywibGVmdCI6NTIuNDIwNTQzNjcwNjU0Mywid2lkdGgiOjQwLjIxMzMxMDI0MTY5OTIyLCJoZWlnaHQiOjEuNjI1NDU4MzU5NzE4MzIyOH19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjczLjAxMDc1NzQ0NjI4OTA2LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3Ljk0NTc4NTUyMjQ2MDk0LCJoZWlnaHQiOjcuODYzNzY0Mjg2MDQxMjZ9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo4MS4zMDg1NTU2MDMwMjczNCwibGVmdCI6NTIuNDIwNTQzNjcwNjU0Mywid2lkdGgiOjQwLjI0MDM3NTUxODc5ODgzLCJoZWlnaHQiOjEuNjI1NDU4MzU5NzE4MzIyOH19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjgyLjkxOTA1OTc1MzQxNzk3LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3LjkzNjQxNjYyNTk3NjU2LCJoZWlnaHQiOjEuNTc4MTMzNTgzMDY4ODQ3N319LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjg0LjQ4NjkwMDMyOTU4OTg0LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3LjkzNDc1NzIzMjY2NjAxNiwiaGVpZ2h0IjoxLjU3ODEzMzU4MzA2ODg0Nzd9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo4Ni4wNTQ3NDg1MzUxNTYyNSwibGVmdCI6NTQuNjk4MzEwODUyMDUwNzgsIndpZHRoIjozNy45Nzg0MzE3MDE2NjAxNTYsImhlaWdodCI6NC43MTM4MTkwMjY5NDcwMjE1fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6OC40MjAyOTI4NTQzMDkwODIsImxlZnQiOjEwLjAyMjE3MjkyNzg1NjQ0NSwid2lkdGgiOjM3Ljk0NjgzMDc0OTUxMTcyLCJoZWlnaHQiOjEuNTc4MTM1MjUxOTk4OTAxNH19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjkuOTg4MTMyNDc2ODA2NjQsImxlZnQiOjEwLjAyMjE3MjkyNzg1NjQ0NSwid2lkdGgiOjEzLjg2MjQ2MTA5MDA4Nzg5LCJoZWlnaHQiOjEuNTc4MTM1MjUxOTk4OTAxNH19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjI0OSJ9fV0%3D.

### Desalination for Agricultural Use

Desalination converts seawater or brackish groundwater into freshwater for irrigation, offering a drought-proof supply in coastal and arid zones, supporting sustainable agriculture amid escalating water demands (Daraz et al., 2025)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlVtYXIiLCJmYW1pbHkiOiJEYXJheiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiLFoHRlZmFuIiwiZmFtaWx5IjoiQm9qbmVjIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllvdW5hcyIsImZhbWlseSI6IktoYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMDc3LTA0NzIiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzOTAvYWdyaWN1bHR1cmUxNTA1MDU1NCIsIm9wZW5hbGV4IjoiVzQ0MDgyMTk4MTEiLCJleGEiOiJodHRwczovL3d3dy5tZHBpLmNvbS8yMDc3LTA0NzIvMTUvNS81NTQifSwidGl0bGUiOiJFbmVyZ3ktRWZmaWNpZW50IFNtYXJ0IElycmlnYXRpb24gVGVjaG5vbG9naWVzOiBBIFBhdGh3YXkgdG8gV2F0ZXIgYW5kIEVuZXJneSBTdXN0YWluYWJpbGl0eSBpbiBBZ3JpY3VsdHVyZSIsImFic3RyYWN0IjoiVGhlIGFncmljdWx0dXJhbCBzZWN0b3IgZmFjZXMgY2hhbGxlbmdlcyBzdWNoIGFzIHdhdGVyIHNjYXJjaXR5LCBlbmVyZ3kgaW5lZmZpY2llbmN5LCBhbmQgZGVjbGluaW5nIHByb2R1Y3Rpdml0eSwgcGFydGljdWxhcmx5IGluIGFyaWQgcmVnaW9ucy4gVHJhZGl0aW9uYWwgaXJyaWdhdGlvbiBtZXRob2RzIGNvbnRyaWJ1dGUgdG8gcmVzb3VyY2UgZGVwbGV0aW9uIGFuZCBlbnZpcm9ubWVudGFsIGltcGFjdHMuIFNvbGFyLXBvd2VyZWQgc21hcnQgaXJyaWdhdGlvbiBzeXN0ZW1zIGludGVncmF0ZSBwcmVjaXNpb24gaXJyaWdhdGlvbiB3aXRoIHJlbmV3YWJsZSBlbmVyZ3ksIGltcHJvdmluZyB3YXRlciB1c2UgYW5kIHByb2R1Y3Rpdml0eS4gSW4gUGFraXN0YW4sIHdoZXJlIGFncmljdWx0dXJlIGNvbnRyaWJ1dGVzIDE5JSBvZiBncm9zcyBkb21lc3RpYyBwcm9kdWN0IGFuZCBlbXBsb3lzIDQwJSBvZiB0aGUgd29ya2ZvcmNlLCB0aGVzZSBjaGFsbGVuZ2VzIGFyZSBzZXZlcmUsIGVzcGVjaWFsbHkgaW4gd2F0ZXItc2NhcmNlIGFyZWFzIGxpa2UgdGhlIENob2xpc3RhbiBEZXNlcnQuIFRoaXMgc3R1ZHkgZXhhbWluZXMgdGhlIGltcGFjdCBvZiBzb2xhci1wb3dlcmVkIHNtYXJ0IGlycmlnYXRpb24gb24gYWdyaWN1bHR1cmFsIHByb2R1Y3Rpdml0eSwgd2F0ZXIgY29uc2VydmF0aW9uLCBhbmQgZW5lcmd5IGVmZmljaWVuY3kgaW4gdGhlIENob2xpc3RhbiBEZXNlcnQuIFVzaW5nIGEgcXVhbnRpdGF0aXZlIGNyb3NzLXNlY3Rpb25hbCBkZXNpZ24sIGRhdGEgd2VyZSBjb2xsZWN0ZWQgZnJvbSAzODQgZmFybWVycyB2aWEgc3RydWN0dXJlZCBxdWVzdGlvbm5haXJlcy4gU3RhdGlzdGljYWwgYW5hbHlzZXMsIGluY2x1ZGluZyBtdWx0aXBsZSBsaW5lYXIgcmVncmVzc2lvbiwgcGFpcmVkIHNhbXBsZSB0LXRlc3RzLCBhbmQgU3RydWN0dXJhbCBFcXVhdGlvbiBNb2RlbGluZyAoU0VNKSwgd2VyZSBjb25kdWN0ZWQuIEZpbmRpbmdzIHNob3cgc2lnbmlmaWNhbnQgaW1wcm92ZW1lbnRzIGluIGNyb3AgeWllbGQgKGZyb20gMy4wIHRvIDQuOCB0b25zL2hlY3RhcmUpIGFuZCByZWR1Y3Rpb25zIGluIHdhdGVyIGFuZCBlbmVyZ3kgY29uc3VtcHRpb24uIFJlZ3Jlc3Npb24gYW5hbHlzaXMgaGlnaGxpZ2h0ZWQgc3Ryb25nIHBvc2l0aXZlIGVmZmVjdHMgb24geWllbGQgYW5kIGVmZmljaWVuY3ksIHdoaWxlIFNFTSBjb25maXJtZWQgcmVkdWNlZCBlbnZpcm9ubWVudGFsIGltcGFjdCBhbmQgb3BlcmF0aW9uYWwgY29zdHMuIFRoZSBzdHVkeSBjb25jbHVkZXMgdGhhdCBzb2xhci1wb3dlcmVkIGlycmlnYXRpb24gZW5oYW5jZXMgcHJvZHVjdGl2aXR5LCBjb25zZXJ2ZXMgcmVzb3VyY2VzLCBhbmQgcHJvbW90ZXMgc3VzdGFpbmFiaWxpdHkuIFBvbGljeW1ha2VycyBzaG91bGQgcHJvdmlkZSBmaW5hbmNpYWwgaW5jZW50aXZlcywgaW52ZXN0IGluIHJlbmV3YWJsZSBpbmZyYXN0cnVjdHVyZSwgYW5kIGltcGxlbWVudCB0cmFpbmluZyBwcm9ncmFtcyB0byBzdXBwb3J0IGFkb3B0aW9uLiBDb2xsYWJvcmF0aXZlIGVmZm9ydHMgYXJlIGVzc2VudGlhbCBmb3Igc3VzdGFpbmFibGUgYWdyaWN1bHR1cmUgaW4gYXJpZCByZWdpb25zLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJpc3N1ZWREYXRlIjoiMjAyNS0wMy0wNSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkFncmljdWx0dXJlIiwiaXNzdWUiOjUsInZvbHVtZSI6MTUsImltcGFjdEZhY3RvciI6IjUuMjYifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjo1NTQsImxhc3RQYWdlIjo1NTR9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiTXVsdGlkaXNjaXBsaW5hcnkgRGlnaXRhbCBQdWJsaXNoaW5nIEluc3RpdHV0ZSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NDgsImNpdGVkQnlDb3VudCI6Mn19LCJxdW90ZSI6IjEuIEludHJvZHVjdGlvbiBUaGUgYWdyaWN1bHR1cmFsIHNlY3RvciwgYSBjb3JuZXJzdG9uZSBvZiBnbG9iYWwgZm9vZCBzZWN1cml0eSBhbmQgZWNvbm9taWMgZGV2ZWxvcG1lbnQsIGZhY2VzIG1vdW50aW5nIGNoYWxsZW5nZXMgaW4gdGhlIDIxc3QgY2VudHVyeS4gQ2xpbWF0ZSBjaGFuZ2UsIHJhcGlkIHBvcHVsYXRpb24gZ3Jvd3RoLCBhbmQgZHdpbmRsaW5nIG5hdHVyYWwgcmVzb3VyY2VzIGhhdmUgaW50ZW5zaWZpZWQgdGhlIG5lZWQgZm9yIHN1c3RhaW5hYmxlIGFncmljdWx0dXJhbCBwcmFjdGljZXMuIEFtb25nIHRoZXNlIGNoYWxsZW5nZXMsIHdhdGVyIHNjYXJjaXR5IGFuZCBlbmVyZ3kgaW5lZmZpY2llbmN5IHN0YW5kIG91dCBhcyBjcml0aWNhbCBiYXJyaWVycyB0byBwcm9kdWN0aXZpdHkgYW5kIHN1c3RhaW5hYmlsaXR5LiBHbG9iYWxseSwgYWdyaWN1bHR1cmUgYWNjb3VudHMgZm9yIGFwcHJveGltYXRlbHkgNzAlIG9mIGZyZXNod2F0ZXIgd2l0aGRyYXdhbHMgYW5kIGlzIG9uZSBvZiB0aGUgbGFyZ2VzdCBjb25zdW1lcnMgb2YgZW5lcmd5IGluIG1hbnkgcmVnaW9ucyBbIDEsIDJdLiBUcmFkaXRpb25hbCBpcnJpZ2F0aW9uIHByYWN0aWNlcywgb2Z0ZW4gaW5lZmZpY2llbnQgYW5kIHJlc291cmNlLWludGVuc2l2ZSwgZXhhY2VyYmF0ZSB0aGVzZSBpc3N1ZXMsIGxlYWRpbmcgdG8gd2F0ZXIgd2FzdGFnZSwgZW5lcmd5IG92ZXJ1c2UsIGFuZCByZWR1Y2VkIGNyb3AgeWllbGRzLiJ9XQ%3D%3D.

Principles of Operation

Reverse osmosis or thermal distillation removes salts, with energy recovery devices enhancing efficiency; treated water blends with conventional sources for optimal salinity.

Advantages and Efficiency Gains

- Unlimited source potential: Scales to meet growing needs;

- Quality control: Produces low-salinity water ideal for sensitive crops (Kushwaha et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjMwLCJzdWJ0eXBlIjp7fSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTWFuaXNoIiwiZmFtaWx5IjoiS3VzaHdhaGEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2hhbmthciIsImZhbWlseSI6IlNpbmdoIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlZpamF5IEthbnQiLCJmYW1pbHkiOiJTaW5naCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTaGFzaGFuayIsImZhbWlseSI6IkR3aXZlZGkifX1dLCJpZHMiOnsiaXNzbiI6WyIyNDU2LTE4NzgiXSwiaXNibiI6W10sImRvaSI6IjEwLjIyMTYxL2lqZWFiLjkyLjI3Iiwib3BlbmFsZXgiOiJXNDM5NjU2MzU4NCJ9LCJ0aXRsZSI6IlByZWNpc2lvbiBGYXJtaW5nOiBBIFJldmlldyBvZiBNZXRob2RzLCBUZWNobm9sb2dpZXMsIGFuZCBGdXR1cmUgUHJvc3BlY3RzIiwiYWJzdHJhY3QiOiJQcmVjaXNpb24gZmFybWluZywgb2Z0ZW4gcmVmZXJyZWQgdG8gYXMgcHJlY2lzaW9uIGFncmljdWx0dXJlLCBpcyBhIGZhcm1pbmcgbWFuYWdlbWVudCBjb25jZXB0IHRoYXQgc2Vla3MgdG8gbWF4aW1pemUgYWdyaWN1bHR1cmFsIG9wZXJhdGlvbnMgdmlhIHRoZSB1c2Ugb2YgdGVjaG5vbG9neSBpbiBvcmRlciB0byBhY2hpZXZlIGdyZWF0ZXIgbGV2ZWxzIG9mIGVmZmljaWVuY3ksIHByb2R1Y3Rpb24sIGFuZCBjb25zZXJ2YXRpb24gb2YgcmVzb3VyY2VzLiBUaGUgcHVycG9zZSBvZiB0aGlzIHN1cnZleSBzdHVkeSBpcyB0byBvZmZlciBhbiBvdmVydmlldyBvZiBwcmVjaXNpb24gZmFybWluZywgaW5jbHVkaW5nIHRvcGljcyBzdWNoIGFzIGl0cyBmdW5kYW1lbnRhbHMsIG1ldGhvZG9sb2dpZXMsIHRlY2hub2xvZ3ksIGFuZCBhcHBsaWNhdGlvbnMuIEl0IGludmVzdGlnYXRlcyB0aGUgcm9sZSB0aGF0IGEgdmFyaWV0eSBvZiB0ZWNobm9sb2dpZXMsIGluY2x1ZGluZyBnbG9iYWwgbmF2aWdhdGlvbiBzYXRlbGxpdGUgc3lzdGVtcyAoR05TUyksIGdlb2dyYXBoaWMgaW5mb3JtYXRpb24gc3lzdGVtcyAoR0lTKSwgcmVtb3RlIHNlbnNpbmcsIGFuZCBzZW5zb3IgdGVjaG5vbG9neSwgcGxheSBpbiBtYWtpbmcgaXQgcG9zc2libGUgdG8gaW1wbGVtZW50IHByZWNpc2lvbiBhZ3JpY3VsdHVyYWwgbWV0aG9kcy4gSW4gYWRkaXRpb24sIGl0IGFkZHJlc3NlcyB0aGUgYWR2YW50YWdlcywgZGlzYWR2YW50YWdlcywgYW5kIHBvdGVudGlhbCBmdXR1cmUgYXBwbGljYXRpb25zIG9mIHByZWNpc2lvbiBmYXJtaW5nIGluIHRoZSBjb250ZXh0IG9mIHNvbHZpbmcgaXNzdWVzIHJlbGF0ZWQgdG8gdGhlIHN1c3RhaW5hYmlsaXR5IG9mIGFncmljdWx0dXJlIGFuZCB0aGUgc2VjdXJpdHkgb2YgZm9vZCBzdXBwbGllcyBvbiBhIHdvcmxkd2lkZSBzY2FsZS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjIxNjEvaWplYWIuOTIuMjciLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vaWplYWIuY29tL3VwbG9hZF9kb2N1bWVudC9pc3N1ZV9maWxlcy8yN0lKRUFCLTEwNDIwMjQ4NC1QcmVjaXNpb25GYXJtaW5nLnBkZiIsImlzc3VlZERhdGUiOiIyMDI0LTAxLTAxIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiSW50ZXJuYXRpb25hbCBKb3VybmFsIG9mIEVudmlyb25tZW50IEFncmljdWx0dXJlIGFuZCBCaW90ZWNobm9sb2d5IiwiaXNzdWUiOjIsInZvbHVtZSI6OSwiaW1wYWN0RmFjdG9yIjoiMC42NSJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjI0MiwibGFzdFBhZ2UiOjI1M319LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJJbmZvZ2FpbiBQdWJsaWNhdGlvbiJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NCwiY2l0ZWRCeUNvdW50Ijo4fX0sInF1b3RlIjoiSUlJLiBCRU5FRklUUyBPRiBQUkVDSVNJT04gSVJSSUdBVElPTiAxLiBJbXByb3ZlZCBXYXRlciBVc2UgRWZmaWNpZW5jeTogUHJlY2lzaW9uIGlycmlnYXRpb24gb3B0aW1pemVzIHdhdGVyIHVzZSBlZmZpY2llbmN5IGJ5IGRlbGl2ZXJpbmcgd2F0ZXIgZGlyZWN0bHkgdG8gdGhlIHJvb3Qgem9uZSBvZiBjcm9wcywgbWluaW1pemluZyBsb3NzZXMgIGR1ZSB0byBldmFwb3JhdGlvbiwgcnVub2ZmLCBhbmQgZGVlcCBwZXJjb2xhdGlvbi4gQnkgIG1hdGNoaW5nIHdhdGVyIGFwcGxpY2F0aW9uIHJhdGVzIHRvIGNyb3Agd2F0ZXIgIHJlcXVpcmVtZW50cywgcHJlY2lzaW9uIGlycmlnYXRpb24gc3lzdGVtcyBlbnN1cmUgdGhhdCAgd2F0ZXIgaXMgdXNlZCBtb3JlIGVmZmljaWVudGx5LCBtYXhpbWl6aW5nIGNyb3AgeWllbGRzICBwZXIgdW5pdCBvZiB3YXRlciBhcHBsaWVkLiAyLiBFbmhhbmNlZCBDcm9wIFBlcmZvcm1hbmNlOiBCeSBwcm92aWRpbmcgY3JvcHMgd2l0aCBvcHRpbWFsIG1vaXN0dXJlIGxldmVscywgcHJlY2lzaW9uIGlycmlnYXRpb24gc3lzdGVtcyBwcm9tb3RlIGhlYWx0aHkgcm9vdCBkZXZlbG9wbWVudCwgYmFsYW5jZWQgZ3Jvd3RoLCAgYW5kIGltcHJvdmVkIGNyb3Agdmlnb3IuIENvbnNpc3RlbnQgc29pbCBtb2lzdHVyZSAgbGV2ZWxzIGhlbHAgcmVkdWNlIHdhdGVyIHN0cmVzcywgaW5jcmVhc2UgbnV0cmllbnQgIHVwdGFrZSwgYW5kIGVuaGFuY2UgY3JvcCByZXNpbGllbmNlIHRvIGVudmlyb25tZW50YWwgIHN0cmVzc2VzIHN1Y2ggYXMgZHJvdWdodCBhbmQgaGVhdC4gMy4gQ29zdCBTYXZpbmdzOiBQcmVjaXNpb24gaXJyaWdhdGlvbiBzeXN0ZW1zIGhlbHAgZmFybWVycyByZWR1Y2Ugd2F0ZXIsIGVuZXJneSwgYW5kIGxhYm9yIGNvc3RzIGFzc29jaWF0ZWQgd2l0aCBpcnJpZ2F0aW9uLiBCeSBvcHRpbWl6aW5nIHdhdGVyIHVzZSBlZmZpY2llbmN5IGFuZCAgbWluaW1pemluZyBpbnB1dCB3YXN0YWdlLCBwcmVjaXNpb24gaXJyaWdhdGlvbiBzeXN0ZW1zICBjYW4gbGVhZCB0byBzaWduaWZpY2FudCBjb3N0IHNhdmluZ3Mgb3ZlciB0aW1lLCAgaW1wcm92aW5nIHRoZSBwcm9maXRhYmlsaXR5IG9mIGFncmljdWx0dXJhbCBvcGVyYXRpb25zLiA0LiBFbnZpcm9ubWVudGFsIFN1c3RhaW5hYmlsaXR5OiAgUHJlY2lzaW9uICBpcnJpZ2F0aW9uIGNvbnRyaWJ1dGVzIHRvICBlbnZpcm9ubWVudGFsICBzdXN0YWluYWJpbGl0eSAgYnkgY29uc2VydmluZyB3YXRlciAgcmVzb3VyY2VzLCAgbWluaW1pemluZyAgd2F0ZXIgcG9sbHV0aW9uLCBhbmQgcmVkdWNpbmcgZ3JlZW5ob3VzZSBnYXMgZW1pc3Npb25zLiBCeSBhZG9wdGluZyBwcmVjaXNpb24gaXJyaWdhdGlvbiBwcmFjdGljZXMsIGZhcm1lcnMgY2FuICBtaW5pbWl6ZSB0aGVpciBlbnZpcm9ubWVudGFsIGZvb3RwcmludCBhbmQgY29udHJpYnV0ZSB0byB0aGUgIGxvbmctdGVybSAgc3VzdGFpbmFiaWxpdHkgIG9mICBhZ3JpY3VsdHVyYWwgcHJvZHVjdGlvbiBzeXN0ZW1zLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjQ0Ljg2NTY0MjU0NzYwNzQyLCJsZWZ0Ijo1My40MDgyNTI3MTYwNjQ0NSwid2lkdGgiOjM4LjI2MTc2ODM0MTA2NDQ1LCJoZWlnaHQiOjEuNjY5MjI3MzYxNjc5MDc3MX19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjQ2Ljg5NzkyMjUxNTg2OTE0LCJsZWZ0Ijo1Mi40MjA1NDM2NzA2NTQzLCJ3aWR0aCI6NDAuMjQwNzM3OTE1MDM5MDYsImhlaWdodCI6MS42MjU0NTQ3ODM0Mzk2MzYyfX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NDguNDkxNzkwNzcxNDg0Mzc1LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3Ljk4MDI3ODAxNTEzNjcyLCJoZWlnaHQiOjExLjAxMzcxMzgzNjY2OTkyMn19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjU5LjkzOTU0MDg2MzAzNzExLCJsZWZ0Ijo1Mi40MjA1NDM2NzA2NTQzLCJ3aWR0aCI6NDAuMjMwMjI0NjA5Mzc1LCJoZWlnaHQiOjEuNjI1NDU0NzgzNDM5NjM2Mn19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjYxLjUzMzQxNjc0ODA0Njg3NSwibGVmdCI6NTQuNjk4MzEwODUyMDUwNzgsIndpZHRoIjozNy45NjMxMjMzMjE1MzMyLCJoZWlnaHQiOjkuNDM1MTcwMTczNjQ1MDJ9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo3MS40MTY4Nzc3NDY1ODIwMywibGVmdCI6NTIuNDIwNTQzNjcwNjU0Mywid2lkdGgiOjQwLjIxMzMxMDI0MTY5OTIyLCJoZWlnaHQiOjEuNjI1NDU4MzU5NzE4MzIyOH19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjczLjAxMDc1NzQ0NjI4OTA2LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3Ljk0NTc4NTUyMjQ2MDk0LCJoZWlnaHQiOjcuODYzNzY0Mjg2MDQxMjZ9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo4MS4zMDg1NTU2MDMwMjczNCwibGVmdCI6NTIuNDIwNTQzNjcwNjU0Mywid2lkdGgiOjQwLjI0MDM3NTUxODc5ODgzLCJoZWlnaHQiOjEuNjI1NDU4MzU5NzE4MzIyOH19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjgyLjkxOTA1OTc1MzQxNzk3LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3LjkzNjQxNjYyNTk3NjU2LCJoZWlnaHQiOjEuNTc4MTMzNTgzMDY4ODQ3N319LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjg0LjQ4NjkwMDMyOTU4OTg0LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3LjkzNDc1NzIzMjY2NjAxNiwiaGVpZ2h0IjoxLjU3ODEzMzU4MzA2ODg0Nzd9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo4Ni4wNTQ3NDg1MzUxNTYyNSwibGVmdCI6NTQuNjk4MzEwODUyMDUwNzgsIndpZHRoIjozNy45Nzg0MzE3MDE2NjAxNTYsImhlaWdodCI6NC43MTM4MTkwMjY5NDcwMjE1fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6OC40MjAyOTI4NTQzMDkwODIsImxlZnQiOjEwLjAyMjE3MjkyNzg1NjQ0NSwid2lkdGgiOjM3Ljk0NjgzMDc0OTUxMTcyLCJoZWlnaHQiOjEuNTc4MTM1MjUxOTk4OTAxNH19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjkuOTg4MTMyNDc2ODA2NjQsImxlZnQiOjEwLjAyMjE3MjkyNzg1NjQ0NSwid2lkdGgiOjEzLjg2MjQ2MTA5MDA4Nzg5LCJoZWlnaHQiOjEuNTc4MTM1MjUxOTk4OTAxNH19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjI0OSJ9fV0%3D;

- Energy optimization: Modern plants achieve 3-4 kWh/m³.

Applications and Implementation

Deployed in Middle East, Australia, and Israel for high-value crops like dates and vegetables; hybrid solar-powered plants reduce costs (Jumintono et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikp1bWludG9ubyIsImZhbWlseSI6Ikp1bWludG9ubyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJOdXJuYXdhdHkiLCJmYW1pbHkiOiJOdXJuYXdhdHkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTnVncm9obyBUcmlzbnUiLCJmYW1pbHkiOiJCcmF0YSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJEaW5pIiwiZmFtaWx5IjoiUmFobWlhdGkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSGFkaSIsImZhbWlseSI6IlN1c2lsbyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBLiBBLiIsImZhbWlseSI6Ikh1c3NlbiJ9fV0sImlkcyI6eyJpc3NuIjpbIjE0MjktNzQyNiIsIjIwODMtNDUzNSJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMjQ0MjUvandsZC4yMDI0LjE1MTc5OCIsIm9wZW5hbGV4IjoiVzQ0MDU0NzcxMDAifSwidGl0bGUiOiJDb21wYXJhdGl2ZSBhbmFseXNpcyBvZiBwcmVjaXNpb24sIGRlZmljaXQsIGFuZCBkcmlwIGlycmlnYXRpb24gZm9yIHdhdGVyIGVmZmljaWVuY3kgaW4gSXJhcSIsImFic3RyYWN0IjoiU3VzdGFpbmFibGUgaXJyaWdhdGlvbiBwcmFjdGljZXMgYXJlIGNyaXRpY2FsIGZvciBhZGRyZXNzaW5nIHRoZSBjaGFsbGVuZ2VzIG9mIHdhdGVyIHNjYXJjaXR5IGFuZCBmb29kIGluc2VjdXJpdHkgaW4gdGhlIGZhY2Ugb2YgY2xpbWF0ZSBjaGFuZ2UgYW5kIGdyb3dpbmcgZ2xvYmFsIGZvb2QgZGVtYW5kLiBUaGlzIHN0dWR5IGludmVzdGlnYXRlZCB0aGUgcG90ZW50aWFsIG9mIHByZWNpc2lvbiBpcnJpZ2F0aW9uIChQSSksIGRlZmljaXQgaXJyaWdhdGlvbiAoREkpLCBhbmQgZHJpcCBpcnJpZ2F0aW9uIChEUkkpIHRlY2huaXF1ZXMgdG8gZW5oYW5jZSBjcm9wIHByb2R1Y3Rpdml0eSBhbmQgd2F0ZXIgdXNlIGVmZmljaWVuY3kgKFdVRSkgY29tcGFyZWQgdG8gY29udmVudGlvbmFsIGZsb29kIGlycmlnYXRpb24gKENGSSkgaW4gdGhyZWUgZGlzdGluY3QgYWdyb2Vjb2xvZ2ljYWwgem9uZXMgb2YgSXJhcS4gRmllbGQgZXhwZXJpbWVudHMgd2VyZSBjb25kdWN0ZWQgdXNpbmcgYSByYW5kb21pc2VkIGNvbXBsZXRlIGJsb2NrIGRlc2lnbiB3aXRoIHdoZWF0LCBtYWl6ZSwgYW5kIHJpY2UgY3JvcHMuIFRoZSByZXN1bHRzIGRlbW9uc3RyYXRlZCB0aGF0IFBJIHNpZ25pZmljYW50bHkgaW5jcmVhc2VkIGNyb3AgZ3Jvd3RoIHBhcmFtZXRlcnMsIGdyYWluIHlpZWxkLCBhbmQgV1VFIGFjcm9zcyBhbGwgem9uZXMsIHdpdGggeWllbGQgaW1wcm92ZW1lbnRzIG9mIDMz4oCTMzglIGFuZCBXVUUgaW5jcmVhc2VzIG9mIDQ24oCTNTElIGluIGNvbnRyYXN0IHdpdGggQ0ZJLiBUaGUgREkgYW5kIERSSSB0cmVhdG1lbnRzIGFsc28gb3V0cGVyZm9ybWVkIENGSSwgYWxiZWl0IHRvIGEgbGVzc2VyIGV4dGVudC4gUmVtb3RlIHNlbnNpbmctZGVyaXZlZCB2ZWdldGF0aW9uIGluZGljZXMgc3Ryb25nbHkgY29ycmVsYXRlZCB3aXRoIGNyb3AgZ3Jvd3RoIHBhcmFtZXRlcnMgYW5kIHlpZWxkLCB3aGlsZSBoeWRyb2xvZ2ljYWwgbW9kZWxsaW5nIHJldmVhbGVkIHJlZHVjZWQgZXZhcG90cmFuc3BpcmF0aW9uIGFuZCBzdXJmYWNlIHJ1bm9mZiB1bmRlciB0aGUgUEkgdHJlYXRtZW50LiBUaGUgc3VzdGFpbmFibGUgaXJyaWdhdGlvbiBwcmFjdGljZXMgcmVzdWx0ZWQgaW4gc3Vic3RhbnRpYWwgd2F0ZXIgc2F2aW5ncyBvZiAyMOKAkzMwJSBjb21wYXJlZCB0byBDRkkuIFRoZXNlIGZpbmRpbmdzIGhpZ2hsaWdodCB0aGUgaW1wb3J0YW5jZSBvZiBhZG9wdGluZyBlZmZpY2llbnQgaXJyaWdhdGlvbiB0ZWNobmlxdWVzLCBhbG9uZyB3aXRoIGEgaG9saXN0aWMgYXBwcm9hY2ggZW5jb21wYXNzaW5nIHRlY2hub2xvZ2ljYWwgaW5ub3ZhdGlvbnMsIGNhcGFjaXR5IGJ1aWxkaW5nLCBhbmQgc3Rha2Vob2xkZXIgZW5nYWdlbWVudCwgdG8gcHJvbW90ZSB3YXRlci0gZWZmaWNpZW50IGFncmljdWx0dXJlIGFuZCBlbnN1cmUgZm9vZCBzZWN1cml0eSBpbiB3YXRlci1zY2FyY2UgcmVnaW9ucy4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjQ0MjUvandsZC4yMDI0LjE1MTc5OCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjI0NDI1L2p3bGQuMjAyNC4xNTE3OTgiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMi0xNCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkpvdXJuYWwgb2YgV2F0ZXIgYW5kIExhbmQgRGV2ZWxvcG1lbnQiLCJpbXBhY3RGYWN0b3IiOiIxLjQ2In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTMwLCJsYXN0UGFnZSI6MTM2fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkRlIEdydXl0ZXIgT3BlbiJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MzIsImNpdGVkQnlDb3VudCI6MX19LCJxdW90ZSI6IkluIHRoaXMgY29udGV4dCwgdGhlIGFkb3B0aW9uIG9mICBzdXN0YWluYWJsZSBpcnJpZ2F0aW9uIHByYWN0aWNlcyBoYXMgZW1lcmdlZCBhcyBhIGNyaXRpY2FsIHN0cmF0ZWd5ICBmb3IgZW5oYW5jaW5nIGZvb2Qgc2VjdXJpdHkgd2hpbGUgcHJvbW90aW5nIHdhdGVyIGNvbnNlcnZhdGlvbiAgaW4gYWdyaWN1bHR1cmFsIGxhbmRzY2FwZXMuICBJcnJpZ2F0aW9uIHBsYXlzIGEgdml0YWwgcm9sZSBpbiBnbG9iYWwgZm9vZCBwcm9kdWN0aW9uLCB3aXRoICBpcnJpZ2F0ZWQgYWdyaWN1bHR1cmUgYWNjb3VudGluZyBmb3IgYXBwcm94aW1hdGVseSA0MCUgb2YgdGhlICBmb29kIHN1cHBseSAoV3JhY2hpZW4gZGUsIFNjaHVsdHogYW5kIEdvbGksIDIwMjE7IFNjaG1pdHQsIFJvc2EgIGFuZCBEYWlseSwgMjAyMikuIEhvd2V2ZXIsIHRyYWRpdGlvbmFsIGlycmlnYXRpb24gbWV0aG9kcywgc3VjaCBhcyAgZmxvb2QgYW5kIGZ1cnJvdyBpcnJpZ2F0aW9uLCBhcmUgb2Z0ZW4gaW5lZmZpY2llbnQsIGxlYWRpbmcgdG8gIHN1YnN0YW50aWFsIHdhdGVyIGxvc3NlcyB0aHJvdWdoIGV2YXBvcmF0aW9uLCBydW5vZmYsIGFuZCBkZWVwICBwZXJjb2xhdGlvbiAoQW1ib21zYSwgMjAyMCkuIE1vcmVvdmVyLCB0aGUgZXhjZXNzaXZlIGFwcGxpY2EtdGlvbiBvZiB3YXRlciBjYW4gcmVzdWx0IGluIHNvaWwgc2FsaW5pc2F0aW9uLCBudXRyaWVudCBsZWFjaGluZywgYW5kICBncm91bmR3YXRlciBkZXBsZXRpb24sIGZ1cnRoZXIgY29tcHJvbWlzaW5nIHRoZSBsb25nLXRlcm0gIHN1c3RhaW5hYmlsaXR5IG9mIGFncmljdWx0dXJhbCBzeXN0ZW1zIChZaW4gZXQgYWwuLCAyMDIyKS4gVG8gYWRkcmVzcyAgdGhlc2UgY2hhbGxlbmdlcywgcmVzZWFyY2hlcnMgYW5kIHByYWN0aXRpb25lcnMgaGF2ZSBiZWVuIGV4cGxvci1pbmcgaW5ub3ZhdGl2ZSBpcnJpZ2F0aW9uIHRlY2huaXF1ZXMgYW5kIHdhdGVyIG1hbmFnZW1lbnQgIHN0cmF0ZWdpZXMgdGhhdCBjYW4gb3B0aW1pc2Ugd2F0ZXIgdXNlIGVmZmljaWVuY3kgKFdVRSkgd2hpbGUgIG1haW50YWluaW5nIG9yIGVuaGFuY2luZyBjcm9wIHByb2R1Y3Rpdml0eS4gIFByZWNpc2lvbiBpcnJpZ2F0aW9uIChQSSkgb2ZmZXJzIGEgcHJvbWlzaW5nIHNvbHV0aW9uLCAgaW52b2x2aW5nIHRoZSB0YXJnZXRlZCBhcHBsaWNhdGlvbiBvZiB3YXRlciBzcGVjaWZpY2FsbHkgdG8gdGhlICBuZWVkcyBvZiBpbmRpdmlkdWFsIHBsYW50cyBvciBjcm9wIHpvbmVzIChaaW5rZXJuYWdlbCBldCBhbC4sIDIwMjApLiAgIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyMi44ODY3MzU5MTYxMzc2OTUsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzQ2MjIxOTIzODI4MTI1LCJoZWlnaHQiOjEuNDUwNTQxMDE5NDM5Njk3M319LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI0LjM1MTEyMzgwOTgxNDQ1MywibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTI3MTYwNjQ0NSwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI1LjcyNDk4NzAzMDAyOTI5NywibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTY1MzA3NjE3MiwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI3LjA5ODg1MDI1MDI0NDE0LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjE2LjQxODE2MTM5MjIxMTkxNCwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI4LjQ3MjcwOTY1NTc2MTcyLCJsZWZ0IjoxMi4zODA4ODAzNTU4MzQ5NjEsIndpZHRoIjozNy4wMTIxODQxNDMwNjY0MDYsImhlaWdodCI6MS4zNjAwMTQ3OTYyNTcwMTl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyOS44NDY1NzI4NzU5NzY1NjIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQwNjQ4NjUxMTIzMDUsImhlaWdodCI6MS4zNjAwMTY3MDM2MDU2NTE5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzEuMjE0MDM2OTQxNTI4MzIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQ0ODU2MjYyMjA3MDMsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzIuNTg3OTAyMDY5MDkxOCwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDYwNTc4OTE4NDU3LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMzLjk2MTc2MTQ3NDYwOTM3NSwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDA0ODQ2MTkxNDA2MjUsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzUuMzM1NjI0Njk0ODI0MjIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzM5NzQ0NTY3ODcxMDk0LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjM2LjcwOTQ4NzkxNTAzOTA2LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjAwMTU5MDcyODc1OTc2NiwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC4wODMzNDczMjA1NTY2NCwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTY1MzA3NjE3MiwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOS40NTcyMTA1NDA3NzE0ODQsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQzMDQ4MDk1NzAzMTI1LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQwLjc0MDU0NzE4MDE3NTc4LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjM0NTQ1NTE2OTY3NzczNCwiaGVpZ2h0IjoxLjQ1MDU0MjgwNzU3OTA0MDV9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Mi4yMDQ5MzMxNjY1MDM5MDYsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMDAzMjUwMTIyMDcwMzEsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDMuNTc4Nzk2Mzg2NzE4NzUsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzM3OTIxMTQyNTc4MTI1LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQ0Ljg2MjEzMzAyNjEyMzA1LCJsZWZ0Ijo5LjA0NzYwMTY5OTgyOTEwMiwid2lkdGgiOjQwLjM0NTMxNDAyNTg3ODkwNiwiaGVpZ2h0IjoxLjQ1MDU0MjgwNzU3OTA0MDV9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ni4zMjY1MTkwMTI0NTExNywibGVmdCI6OS4wNDc2MDE2OTk4MjkxMDIsIndpZHRoIjoyNy45Njc0NDcyODA4ODM3OSwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ny43MDAzODIyMzI2NjYwMTYsImxlZnQiOjEyLjM4MDg3ODQ0ODQ4NjMyOCwid2lkdGgiOjM3LjAzNDc4NjIyNDM2NTIzNCwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0OS4wNzQyNDU0NTI4ODA4NiwibGVmdCI6OS4wNDc2MDE2OTk4MjkxMDIsIndpZHRoIjo0MC4zNDA2NDQ4MzY0MjU3OCwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1MC4zNTc1ODIwOTIyODUxNTYsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzQ0ODUyNDQ3NTA5NzY2LCJoZWlnaHQiOjEuNDUwNTQyODA3NTc5MDQwNX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEzMSJ9fV0%3D.

### Deficit Irrigation and Partial Root-Zone Drying

Deficit irrigation applies controlled water deficits to maximize water use efficiency, while partial root-zone drying alternates wetting zones to optimize physiology, achieving substantial savings over traditional methods (Jumintono et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikp1bWludG9ubyIsImZhbWlseSI6Ikp1bWludG9ubyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJOdXJuYXdhdHkiLCJmYW1pbHkiOiJOdXJuYXdhdHkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTnVncm9obyBUcmlzbnUiLCJmYW1pbHkiOiJCcmF0YSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJEaW5pIiwiZmFtaWx5IjoiUmFobWlhdGkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSGFkaSIsImZhbWlseSI6IlN1c2lsbyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBLiBBLiIsImZhbWlseSI6Ikh1c3NlbiJ9fV0sImlkcyI6eyJpc3NuIjpbIjE0MjktNzQyNiIsIjIwODMtNDUzNSJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMjQ0MjUvandsZC4yMDI0LjE1MTc5OCIsIm9wZW5hbGV4IjoiVzQ0MDU0NzcxMDAifSwidGl0bGUiOiJDb21wYXJhdGl2ZSBhbmFseXNpcyBvZiBwcmVjaXNpb24sIGRlZmljaXQsIGFuZCBkcmlwIGlycmlnYXRpb24gZm9yIHdhdGVyIGVmZmljaWVuY3kgaW4gSXJhcSIsImFic3RyYWN0IjoiU3VzdGFpbmFibGUgaXJyaWdhdGlvbiBwcmFjdGljZXMgYXJlIGNyaXRpY2FsIGZvciBhZGRyZXNzaW5nIHRoZSBjaGFsbGVuZ2VzIG9mIHdhdGVyIHNjYXJjaXR5IGFuZCBmb29kIGluc2VjdXJpdHkgaW4gdGhlIGZhY2Ugb2YgY2xpbWF0ZSBjaGFuZ2UgYW5kIGdyb3dpbmcgZ2xvYmFsIGZvb2QgZGVtYW5kLiBUaGlzIHN0dWR5IGludmVzdGlnYXRlZCB0aGUgcG90ZW50aWFsIG9mIHByZWNpc2lvbiBpcnJpZ2F0aW9uIChQSSksIGRlZmljaXQgaXJyaWdhdGlvbiAoREkpLCBhbmQgZHJpcCBpcnJpZ2F0aW9uIChEUkkpIHRlY2huaXF1ZXMgdG8gZW5oYW5jZSBjcm9wIHByb2R1Y3Rpdml0eSBhbmQgd2F0ZXIgdXNlIGVmZmljaWVuY3kgKFdVRSkgY29tcGFyZWQgdG8gY29udmVudGlvbmFsIGZsb29kIGlycmlnYXRpb24gKENGSSkgaW4gdGhyZWUgZGlzdGluY3QgYWdyb2Vjb2xvZ2ljYWwgem9uZXMgb2YgSXJhcS4gRmllbGQgZXhwZXJpbWVudHMgd2VyZSBjb25kdWN0ZWQgdXNpbmcgYSByYW5kb21pc2VkIGNvbXBsZXRlIGJsb2NrIGRlc2lnbiB3aXRoIHdoZWF0LCBtYWl6ZSwgYW5kIHJpY2UgY3JvcHMuIFRoZSByZXN1bHRzIGRlbW9uc3RyYXRlZCB0aGF0IFBJIHNpZ25pZmljYW50bHkgaW5jcmVhc2VkIGNyb3AgZ3Jvd3RoIHBhcmFtZXRlcnMsIGdyYWluIHlpZWxkLCBhbmQgV1VFIGFjcm9zcyBhbGwgem9uZXMsIHdpdGggeWllbGQgaW1wcm92ZW1lbnRzIG9mIDMz4oCTMzglIGFuZCBXVUUgaW5jcmVhc2VzIG9mIDQ24oCTNTElIGluIGNvbnRyYXN0IHdpdGggQ0ZJLiBUaGUgREkgYW5kIERSSSB0cmVhdG1lbnRzIGFsc28gb3V0cGVyZm9ybWVkIENGSSwgYWxiZWl0IHRvIGEgbGVzc2VyIGV4dGVudC4gUmVtb3RlIHNlbnNpbmctZGVyaXZlZCB2ZWdldGF0aW9uIGluZGljZXMgc3Ryb25nbHkgY29ycmVsYXRlZCB3aXRoIGNyb3AgZ3Jvd3RoIHBhcmFtZXRlcnMgYW5kIHlpZWxkLCB3aGlsZSBoeWRyb2xvZ2ljYWwgbW9kZWxsaW5nIHJldmVhbGVkIHJlZHVjZWQgZXZhcG90cmFuc3BpcmF0aW9uIGFuZCBzdXJmYWNlIHJ1bm9mZiB1bmRlciB0aGUgUEkgdHJlYXRtZW50LiBUaGUgc3VzdGFpbmFibGUgaXJyaWdhdGlvbiBwcmFjdGljZXMgcmVzdWx0ZWQgaW4gc3Vic3RhbnRpYWwgd2F0ZXIgc2F2aW5ncyBvZiAyMOKAkzMwJSBjb21wYXJlZCB0byBDRkkuIFRoZXNlIGZpbmRpbmdzIGhpZ2hsaWdodCB0aGUgaW1wb3J0YW5jZSBvZiBhZG9wdGluZyBlZmZpY2llbnQgaXJyaWdhdGlvbiB0ZWNobmlxdWVzLCBhbG9uZyB3aXRoIGEgaG9saXN0aWMgYXBwcm9hY2ggZW5jb21wYXNzaW5nIHRlY2hub2xvZ2ljYWwgaW5ub3ZhdGlvbnMsIGNhcGFjaXR5IGJ1aWxkaW5nLCBhbmQgc3Rha2Vob2xkZXIgZW5nYWdlbWVudCwgdG8gcHJvbW90ZSB3YXRlci0gZWZmaWNpZW50IGFncmljdWx0dXJlIGFuZCBlbnN1cmUgZm9vZCBzZWN1cml0eSBpbiB3YXRlci1zY2FyY2UgcmVnaW9ucy4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjQ0MjUvandsZC4yMDI0LjE1MTc5OCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjI0NDI1L2p3bGQuMjAyNC4xNTE3OTgiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMi0xNCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkpvdXJuYWwgb2YgV2F0ZXIgYW5kIExhbmQgRGV2ZWxvcG1lbnQiLCJpbXBhY3RGYWN0b3IiOiIxLjQ2In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTMwLCJsYXN0UGFnZSI6MTM2fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkRlIEdydXl0ZXIgT3BlbiJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MzIsImNpdGVkQnlDb3VudCI6MX19LCJxdW90ZSI6IkluIHRoaXMgY29udGV4dCwgdGhlIGFkb3B0aW9uIG9mICBzdXN0YWluYWJsZSBpcnJpZ2F0aW9uIHByYWN0aWNlcyBoYXMgZW1lcmdlZCBhcyBhIGNyaXRpY2FsIHN0cmF0ZWd5ICBmb3IgZW5oYW5jaW5nIGZvb2Qgc2VjdXJpdHkgd2hpbGUgcHJvbW90aW5nIHdhdGVyIGNvbnNlcnZhdGlvbiAgaW4gYWdyaWN1bHR1cmFsIGxhbmRzY2FwZXMuICBJcnJpZ2F0aW9uIHBsYXlzIGEgdml0YWwgcm9sZSBpbiBnbG9iYWwgZm9vZCBwcm9kdWN0aW9uLCB3aXRoICBpcnJpZ2F0ZWQgYWdyaWN1bHR1cmUgYWNjb3VudGluZyBmb3IgYXBwcm94aW1hdGVseSA0MCUgb2YgdGhlICBmb29kIHN1cHBseSAoV3JhY2hpZW4gZGUsIFNjaHVsdHogYW5kIEdvbGksIDIwMjE7IFNjaG1pdHQsIFJvc2EgIGFuZCBEYWlseSwgMjAyMikuIEhvd2V2ZXIsIHRyYWRpdGlvbmFsIGlycmlnYXRpb24gbWV0aG9kcywgc3VjaCBhcyAgZmxvb2QgYW5kIGZ1cnJvdyBpcnJpZ2F0aW9uLCBhcmUgb2Z0ZW4gaW5lZmZpY2llbnQsIGxlYWRpbmcgdG8gIHN1YnN0YW50aWFsIHdhdGVyIGxvc3NlcyB0aHJvdWdoIGV2YXBvcmF0aW9uLCBydW5vZmYsIGFuZCBkZWVwICBwZXJjb2xhdGlvbiAoQW1ib21zYSwgMjAyMCkuIE1vcmVvdmVyLCB0aGUgZXhjZXNzaXZlIGFwcGxpY2EtdGlvbiBvZiB3YXRlciBjYW4gcmVzdWx0IGluIHNvaWwgc2FsaW5pc2F0aW9uLCBudXRyaWVudCBsZWFjaGluZywgYW5kICBncm91bmR3YXRlciBkZXBsZXRpb24sIGZ1cnRoZXIgY29tcHJvbWlzaW5nIHRoZSBsb25nLXRlcm0gIHN1c3RhaW5hYmlsaXR5IG9mIGFncmljdWx0dXJhbCBzeXN0ZW1zIChZaW4gZXQgYWwuLCAyMDIyKS4gVG8gYWRkcmVzcyAgdGhlc2UgY2hhbGxlbmdlcywgcmVzZWFyY2hlcnMgYW5kIHByYWN0aXRpb25lcnMgaGF2ZSBiZWVuIGV4cGxvci1pbmcgaW5ub3ZhdGl2ZSBpcnJpZ2F0aW9uIHRlY2huaXF1ZXMgYW5kIHdhdGVyIG1hbmFnZW1lbnQgIHN0cmF0ZWdpZXMgdGhhdCBjYW4gb3B0aW1pc2Ugd2F0ZXIgdXNlIGVmZmljaWVuY3kgKFdVRSkgd2hpbGUgIG1haW50YWluaW5nIG9yIGVuaGFuY2luZyBjcm9wIHByb2R1Y3Rpdml0eS4gIFByZWNpc2lvbiBpcnJpZ2F0aW9uIChQSSkgb2ZmZXJzIGEgcHJvbWlzaW5nIHNvbHV0aW9uLCAgaW52b2x2aW5nIHRoZSB0YXJnZXRlZCBhcHBsaWNhdGlvbiBvZiB3YXRlciBzcGVjaWZpY2FsbHkgdG8gdGhlICBuZWVkcyBvZiBpbmRpdmlkdWFsIHBsYW50cyBvciBjcm9wIHpvbmVzIChaaW5rZXJuYWdlbCBldCBhbC4sIDIwMjApLiAgIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyMi44ODY3MzU5MTYxMzc2OTUsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzQ2MjIxOTIzODI4MTI1LCJoZWlnaHQiOjEuNDUwNTQxMDE5NDM5Njk3M319LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI0LjM1MTEyMzgwOTgxNDQ1MywibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTI3MTYwNjQ0NSwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI1LjcyNDk4NzAzMDAyOTI5NywibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTY1MzA3NjE3MiwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI3LjA5ODg1MDI1MDI0NDE0LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjE2LjQxODE2MTM5MjIxMTkxNCwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI4LjQ3MjcwOTY1NTc2MTcyLCJsZWZ0IjoxMi4zODA4ODAzNTU4MzQ5NjEsIndpZHRoIjozNy4wMTIxODQxNDMwNjY0MDYsImhlaWdodCI6MS4zNjAwMTQ3OTYyNTcwMTl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyOS44NDY1NzI4NzU5NzY1NjIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQwNjQ4NjUxMTIzMDUsImhlaWdodCI6MS4zNjAwMTY3MDM2MDU2NTE5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzEuMjE0MDM2OTQxNTI4MzIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQ0ODU2MjYyMjA3MDMsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzIuNTg3OTAyMDY5MDkxOCwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDYwNTc4OTE4NDU3LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMzLjk2MTc2MTQ3NDYwOTM3NSwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDA0ODQ2MTkxNDA2MjUsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzUuMzM1NjI0Njk0ODI0MjIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzM5NzQ0NTY3ODcxMDk0LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjM2LjcwOTQ4NzkxNTAzOTA2LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjAwMTU5MDcyODc1OTc2NiwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC4wODMzNDczMjA1NTY2NCwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTY1MzA3NjE3MiwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOS40NTcyMTA1NDA3NzE0ODQsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQzMDQ4MDk1NzAzMTI1LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQwLjc0MDU0NzE4MDE3NTc4LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjM0NTQ1NTE2OTY3NzczNCwiaGVpZ2h0IjoxLjQ1MDU0MjgwNzU3OTA0MDV9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Mi4yMDQ5MzMxNjY1MDM5MDYsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMDAzMjUwMTIyMDcwMzEsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDMuNTc4Nzk2Mzg2NzE4NzUsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzM3OTIxMTQyNTc4MTI1LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQ0Ljg2MjEzMzAyNjEyMzA1LCJsZWZ0Ijo5LjA0NzYwMTY5OTgyOTEwMiwid2lkdGgiOjQwLjM0NTMxNDAyNTg3ODkwNiwiaGVpZ2h0IjoxLjQ1MDU0MjgwNzU3OTA0MDV9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ni4zMjY1MTkwMTI0NTExNywibGVmdCI6OS4wNDc2MDE2OTk4MjkxMDIsIndpZHRoIjoyNy45Njc0NDcyODA4ODM3OSwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ny43MDAzODIyMzI2NjYwMTYsImxlZnQiOjEyLjM4MDg3ODQ0ODQ4NjMyOCwid2lkdGgiOjM3LjAzNDc4NjIyNDM2NTIzNCwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0OS4wNzQyNDU0NTI4ODA4NiwibGVmdCI6OS4wNDc2MDE2OTk4MjkxMDIsIndpZHRoIjo0MC4zNDA2NDQ4MzY0MjU3OCwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1MC4zNTc1ODIwOTIyODUxNTYsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzQ0ODUyNDQ3NTA5NzY2LCJoZWlnaHQiOjEuNDUwNTQyODA3NTc5MDQwNX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEzMSJ9fV0%3D.

Principles of Operation

Scheduling delivers 60-80% of full ET requirements, with PRD cycling irrigation between root halves to induce hormonal signals (abscisic acid) for stomatal control and deeper rooting.

Advantages and Efficiency Gains

- WUE boost: 20-40% water savings with minimal yield loss (Jumintono et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikp1bWludG9ubyIsImZhbWlseSI6Ikp1bWludG9ubyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJOdXJuYXdhdHkiLCJmYW1pbHkiOiJOdXJuYXdhdHkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTnVncm9obyBUcmlzbnUiLCJmYW1pbHkiOiJCcmF0YSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJEaW5pIiwiZmFtaWx5IjoiUmFobWlhdGkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSGFkaSIsImZhbWlseSI6IlN1c2lsbyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBLiBBLiIsImZhbWlseSI6Ikh1c3NlbiJ9fV0sImlkcyI6eyJpc3NuIjpbIjE0MjktNzQyNiIsIjIwODMtNDUzNSJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMjQ0MjUvandsZC4yMDI0LjE1MTc5OCIsIm9wZW5hbGV4IjoiVzQ0MDU0NzcxMDAifSwidGl0bGUiOiJDb21wYXJhdGl2ZSBhbmFseXNpcyBvZiBwcmVjaXNpb24sIGRlZmljaXQsIGFuZCBkcmlwIGlycmlnYXRpb24gZm9yIHdhdGVyIGVmZmljaWVuY3kgaW4gSXJhcSIsImFic3RyYWN0IjoiU3VzdGFpbmFibGUgaXJyaWdhdGlvbiBwcmFjdGljZXMgYXJlIGNyaXRpY2FsIGZvciBhZGRyZXNzaW5nIHRoZSBjaGFsbGVuZ2VzIG9mIHdhdGVyIHNjYXJjaXR5IGFuZCBmb29kIGluc2VjdXJpdHkgaW4gdGhlIGZhY2Ugb2YgY2xpbWF0ZSBjaGFuZ2UgYW5kIGdyb3dpbmcgZ2xvYmFsIGZvb2QgZGVtYW5kLiBUaGlzIHN0dWR5IGludmVzdGlnYXRlZCB0aGUgcG90ZW50aWFsIG9mIHByZWNpc2lvbiBpcnJpZ2F0aW9uIChQSSksIGRlZmljaXQgaXJyaWdhdGlvbiAoREkpLCBhbmQgZHJpcCBpcnJpZ2F0aW9uIChEUkkpIHRlY2huaXF1ZXMgdG8gZW5oYW5jZSBjcm9wIHByb2R1Y3Rpdml0eSBhbmQgd2F0ZXIgdXNlIGVmZmljaWVuY3kgKFdVRSkgY29tcGFyZWQgdG8gY29udmVudGlvbmFsIGZsb29kIGlycmlnYXRpb24gKENGSSkgaW4gdGhyZWUgZGlzdGluY3QgYWdyb2Vjb2xvZ2ljYWwgem9uZXMgb2YgSXJhcS4gRmllbGQgZXhwZXJpbWVudHMgd2VyZSBjb25kdWN0ZWQgdXNpbmcgYSByYW5kb21pc2VkIGNvbXBsZXRlIGJsb2NrIGRlc2lnbiB3aXRoIHdoZWF0LCBtYWl6ZSwgYW5kIHJpY2UgY3JvcHMuIFRoZSByZXN1bHRzIGRlbW9uc3RyYXRlZCB0aGF0IFBJIHNpZ25pZmljYW50bHkgaW5jcmVhc2VkIGNyb3AgZ3Jvd3RoIHBhcmFtZXRlcnMsIGdyYWluIHlpZWxkLCBhbmQgV1VFIGFjcm9zcyBhbGwgem9uZXMsIHdpdGggeWllbGQgaW1wcm92ZW1lbnRzIG9mIDMz4oCTMzglIGFuZCBXVUUgaW5jcmVhc2VzIG9mIDQ24oCTNTElIGluIGNvbnRyYXN0IHdpdGggQ0ZJLiBUaGUgREkgYW5kIERSSSB0cmVhdG1lbnRzIGFsc28gb3V0cGVyZm9ybWVkIENGSSwgYWxiZWl0IHRvIGEgbGVzc2VyIGV4dGVudC4gUmVtb3RlIHNlbnNpbmctZGVyaXZlZCB2ZWdldGF0aW9uIGluZGljZXMgc3Ryb25nbHkgY29ycmVsYXRlZCB3aXRoIGNyb3AgZ3Jvd3RoIHBhcmFtZXRlcnMgYW5kIHlpZWxkLCB3aGlsZSBoeWRyb2xvZ2ljYWwgbW9kZWxsaW5nIHJldmVhbGVkIHJlZHVjZWQgZXZhcG90cmFuc3BpcmF0aW9uIGFuZCBzdXJmYWNlIHJ1bm9mZiB1bmRlciB0aGUgUEkgdHJlYXRtZW50LiBUaGUgc3VzdGFpbmFibGUgaXJyaWdhdGlvbiBwcmFjdGljZXMgcmVzdWx0ZWQgaW4gc3Vic3RhbnRpYWwgd2F0ZXIgc2F2aW5ncyBvZiAyMOKAkzMwJSBjb21wYXJlZCB0byBDRkkuIFRoZXNlIGZpbmRpbmdzIGhpZ2hsaWdodCB0aGUgaW1wb3J0YW5jZSBvZiBhZG9wdGluZyBlZmZpY2llbnQgaXJyaWdhdGlvbiB0ZWNobmlxdWVzLCBhbG9uZyB3aXRoIGEgaG9saXN0aWMgYXBwcm9hY2ggZW5jb21wYXNzaW5nIHRlY2hub2xvZ2ljYWwgaW5ub3ZhdGlvbnMsIGNhcGFjaXR5IGJ1aWxkaW5nLCBhbmQgc3Rha2Vob2xkZXIgZW5nYWdlbWVudCwgdG8gcHJvbW90ZSB3YXRlci0gZWZmaWNpZW50IGFncmljdWx0dXJlIGFuZCBlbnN1cmUgZm9vZCBzZWN1cml0eSBpbiB3YXRlci1zY2FyY2UgcmVnaW9ucy4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjQ0MjUvandsZC4yMDI0LjE1MTc5OCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjI0NDI1L2p3bGQuMjAyNC4xNTE3OTgiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMi0xNCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkpvdXJuYWwgb2YgV2F0ZXIgYW5kIExhbmQgRGV2ZWxvcG1lbnQiLCJpbXBhY3RGYWN0b3IiOiIxLjQ2In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTMwLCJsYXN0UGFnZSI6MTM2fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkRlIEdydXl0ZXIgT3BlbiJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MzIsImNpdGVkQnlDb3VudCI6MX19LCJxdW90ZSI6IkluIHRoaXMgY29udGV4dCwgdGhlIGFkb3B0aW9uIG9mICBzdXN0YWluYWJsZSBpcnJpZ2F0aW9uIHByYWN0aWNlcyBoYXMgZW1lcmdlZCBhcyBhIGNyaXRpY2FsIHN0cmF0ZWd5ICBmb3IgZW5oYW5jaW5nIGZvb2Qgc2VjdXJpdHkgd2hpbGUgcHJvbW90aW5nIHdhdGVyIGNvbnNlcnZhdGlvbiAgaW4gYWdyaWN1bHR1cmFsIGxhbmRzY2FwZXMuICBJcnJpZ2F0aW9uIHBsYXlzIGEgdml0YWwgcm9sZSBpbiBnbG9iYWwgZm9vZCBwcm9kdWN0aW9uLCB3aXRoICBpcnJpZ2F0ZWQgYWdyaWN1bHR1cmUgYWNjb3VudGluZyBmb3IgYXBwcm94aW1hdGVseSA0MCUgb2YgdGhlICBmb29kIHN1cHBseSAoV3JhY2hpZW4gZGUsIFNjaHVsdHogYW5kIEdvbGksIDIwMjE7IFNjaG1pdHQsIFJvc2EgIGFuZCBEYWlseSwgMjAyMikuIEhvd2V2ZXIsIHRyYWRpdGlvbmFsIGlycmlnYXRpb24gbWV0aG9kcywgc3VjaCBhcyAgZmxvb2QgYW5kIGZ1cnJvdyBpcnJpZ2F0aW9uLCBhcmUgb2Z0ZW4gaW5lZmZpY2llbnQsIGxlYWRpbmcgdG8gIHN1YnN0YW50aWFsIHdhdGVyIGxvc3NlcyB0aHJvdWdoIGV2YXBvcmF0aW9uLCBydW5vZmYsIGFuZCBkZWVwICBwZXJjb2xhdGlvbiAoQW1ib21zYSwgMjAyMCkuIE1vcmVvdmVyLCB0aGUgZXhjZXNzaXZlIGFwcGxpY2EtdGlvbiBvZiB3YXRlciBjYW4gcmVzdWx0IGluIHNvaWwgc2FsaW5pc2F0aW9uLCBudXRyaWVudCBsZWFjaGluZywgYW5kICBncm91bmR3YXRlciBkZXBsZXRpb24sIGZ1cnRoZXIgY29tcHJvbWlzaW5nIHRoZSBsb25nLXRlcm0gIHN1c3RhaW5hYmlsaXR5IG9mIGFncmljdWx0dXJhbCBzeXN0ZW1zIChZaW4gZXQgYWwuLCAyMDIyKS4gVG8gYWRkcmVzcyAgdGhlc2UgY2hhbGxlbmdlcywgcmVzZWFyY2hlcnMgYW5kIHByYWN0aXRpb25lcnMgaGF2ZSBiZWVuIGV4cGxvci1pbmcgaW5ub3ZhdGl2ZSBpcnJpZ2F0aW9uIHRlY2huaXF1ZXMgYW5kIHdhdGVyIG1hbmFnZW1lbnQgIHN0cmF0ZWdpZXMgdGhhdCBjYW4gb3B0aW1pc2Ugd2F0ZXIgdXNlIGVmZmljaWVuY3kgKFdVRSkgd2hpbGUgIG1haW50YWluaW5nIG9yIGVuaGFuY2luZyBjcm9wIHByb2R1Y3Rpdml0eS4gIFByZWNpc2lvbiBpcnJpZ2F0aW9uIChQSSkgb2ZmZXJzIGEgcHJvbWlzaW5nIHNvbHV0aW9uLCAgaW52b2x2aW5nIHRoZSB0YXJnZXRlZCBhcHBsaWNhdGlvbiBvZiB3YXRlciBzcGVjaWZpY2FsbHkgdG8gdGhlICBuZWVkcyBvZiBpbmRpdmlkdWFsIHBsYW50cyBvciBjcm9wIHpvbmVzIChaaW5rZXJuYWdlbCBldCBhbC4sIDIwMjApLiAgIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyMi44ODY3MzU5MTYxMzc2OTUsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzQ2MjIxOTIzODI4MTI1LCJoZWlnaHQiOjEuNDUwNTQxMDE5NDM5Njk3M319LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI0LjM1MTEyMzgwOTgxNDQ1MywibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTI3MTYwNjQ0NSwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI1LjcyNDk4NzAzMDAyOTI5NywibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTY1MzA3NjE3MiwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI3LjA5ODg1MDI1MDI0NDE0LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjE2LjQxODE2MTM5MjIxMTkxNCwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI4LjQ3MjcwOTY1NTc2MTcyLCJsZWZ0IjoxMi4zODA4ODAzNTU4MzQ5NjEsIndpZHRoIjozNy4wMTIxODQxNDMwNjY0MDYsImhlaWdodCI6MS4zNjAwMTQ3OTYyNTcwMTl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyOS44NDY1NzI4NzU5NzY1NjIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQwNjQ4NjUxMTIzMDUsImhlaWdodCI6MS4zNjAwMTY3MDM2MDU2NTE5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzEuMjE0MDM2OTQxNTI4MzIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQ0ODU2MjYyMjA3MDMsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzIuNTg3OTAyMDY5MDkxOCwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDYwNTc4OTE4NDU3LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMzLjk2MTc2MTQ3NDYwOTM3NSwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDA0ODQ2MTkxNDA2MjUsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzUuMzM1NjI0Njk0ODI0MjIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzM5NzQ0NTY3ODcxMDk0LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjM2LjcwOTQ4NzkxNTAzOTA2LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjAwMTU5MDcyODc1OTc2NiwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC4wODMzNDczMjA1NTY2NCwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTY1MzA3NjE3MiwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOS40NTcyMTA1NDA3NzE0ODQsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQzMDQ4MDk1NzAzMTI1LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQwLjc0MDU0NzE4MDE3NTc4LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjM0NTQ1NTE2OTY3NzczNCwiaGVpZ2h0IjoxLjQ1MDU0MjgwNzU3OTA0MDV9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Mi4yMDQ5MzMxNjY1MDM5MDYsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMDAzMjUwMTIyMDcwMzEsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDMuNTc4Nzk2Mzg2NzE4NzUsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzM3OTIxMTQyNTc4MTI1LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQ0Ljg2MjEzMzAyNjEyMzA1LCJsZWZ0Ijo5LjA0NzYwMTY5OTgyOTEwMiwid2lkdGgiOjQwLjM0NTMxNDAyNTg3ODkwNiwiaGVpZ2h0IjoxLjQ1MDU0MjgwNzU3OTA0MDV9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ni4zMjY1MTkwMTI0NTExNywibGVmdCI6OS4wNDc2MDE2OTk4MjkxMDIsIndpZHRoIjoyNy45Njc0NDcyODA4ODM3OSwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ny43MDAzODIyMzI2NjYwMTYsImxlZnQiOjEyLjM4MDg3ODQ0ODQ4NjMyOCwid2lkdGgiOjM3LjAzNDc4NjIyNDM2NTIzNCwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0OS4wNzQyNDU0NTI4ODA4NiwibGVmdCI6OS4wNDc2MDE2OTk4MjkxMDIsIndpZHRoIjo0MC4zNDA2NDQ4MzY0MjU3OCwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1MC4zNTc1ODIwOTIyODUxNTYsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzQ0ODUyNDQ3NTA5NzY2LCJoZWlnaHQiOjEuNDUwNTQyODA3NTc5MDQwNX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEzMSJ9fV0%3D;

- Quality enhancement: Improves fruit flavor and color;

- Stress tolerance: Builds crop resilience (Kushwaha et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjMwLCJzdWJ0eXBlIjp7fSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTWFuaXNoIiwiZmFtaWx5IjoiS3VzaHdhaGEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2hhbmthciIsImZhbWlseSI6IlNpbmdoIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlZpamF5IEthbnQiLCJmYW1pbHkiOiJTaW5naCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTaGFzaGFuayIsImZhbWlseSI6IkR3aXZlZGkifX1dLCJpZHMiOnsiaXNzbiI6WyIyNDU2LTE4NzgiXSwiaXNibiI6W10sImRvaSI6IjEwLjIyMTYxL2lqZWFiLjkyLjI3Iiwib3BlbmFsZXgiOiJXNDM5NjU2MzU4NCJ9LCJ0aXRsZSI6IlByZWNpc2lvbiBGYXJtaW5nOiBBIFJldmlldyBvZiBNZXRob2RzLCBUZWNobm9sb2dpZXMsIGFuZCBGdXR1cmUgUHJvc3BlY3RzIiwiYWJzdHJhY3QiOiJQcmVjaXNpb24gZmFybWluZywgb2Z0ZW4gcmVmZXJyZWQgdG8gYXMgcHJlY2lzaW9uIGFncmljdWx0dXJlLCBpcyBhIGZhcm1pbmcgbWFuYWdlbWVudCBjb25jZXB0IHRoYXQgc2Vla3MgdG8gbWF4aW1pemUgYWdyaWN1bHR1cmFsIG9wZXJhdGlvbnMgdmlhIHRoZSB1c2Ugb2YgdGVjaG5vbG9neSBpbiBvcmRlciB0byBhY2hpZXZlIGdyZWF0ZXIgbGV2ZWxzIG9mIGVmZmljaWVuY3ksIHByb2R1Y3Rpb24sIGFuZCBjb25zZXJ2YXRpb24gb2YgcmVzb3VyY2VzLiBUaGUgcHVycG9zZSBvZiB0aGlzIHN1cnZleSBzdHVkeSBpcyB0byBvZmZlciBhbiBvdmVydmlldyBvZiBwcmVjaXNpb24gZmFybWluZywgaW5jbHVkaW5nIHRvcGljcyBzdWNoIGFzIGl0cyBmdW5kYW1lbnRhbHMsIG1ldGhvZG9sb2dpZXMsIHRlY2hub2xvZ3ksIGFuZCBhcHBsaWNhdGlvbnMuIEl0IGludmVzdGlnYXRlcyB0aGUgcm9sZSB0aGF0IGEgdmFyaWV0eSBvZiB0ZWNobm9sb2dpZXMsIGluY2x1ZGluZyBnbG9iYWwgbmF2aWdhdGlvbiBzYXRlbGxpdGUgc3lzdGVtcyAoR05TUyksIGdlb2dyYXBoaWMgaW5mb3JtYXRpb24gc3lzdGVtcyAoR0lTKSwgcmVtb3RlIHNlbnNpbmcsIGFuZCBzZW5zb3IgdGVjaG5vbG9neSwgcGxheSBpbiBtYWtpbmcgaXQgcG9zc2libGUgdG8gaW1wbGVtZW50IHByZWNpc2lvbiBhZ3JpY3VsdHVyYWwgbWV0aG9kcy4gSW4gYWRkaXRpb24sIGl0IGFkZHJlc3NlcyB0aGUgYWR2YW50YWdlcywgZGlzYWR2YW50YWdlcywgYW5kIHBvdGVudGlhbCBmdXR1cmUgYXBwbGljYXRpb25zIG9mIHByZWNpc2lvbiBmYXJtaW5nIGluIHRoZSBjb250ZXh0IG9mIHNvbHZpbmcgaXNzdWVzIHJlbGF0ZWQgdG8gdGhlIHN1c3RhaW5hYmlsaXR5IG9mIGFncmljdWx0dXJlIGFuZCB0aGUgc2VjdXJpdHkgb2YgZm9vZCBzdXBwbGllcyBvbiBhIHdvcmxkd2lkZSBzY2FsZS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjIxNjEvaWplYWIuOTIuMjciLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vaWplYWIuY29tL3VwbG9hZF9kb2N1bWVudC9pc3N1ZV9maWxlcy8yN0lKRUFCLTEwNDIwMjQ4NC1QcmVjaXNpb25GYXJtaW5nLnBkZiIsImlzc3VlZERhdGUiOiIyMDI0LTAxLTAxIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiSW50ZXJuYXRpb25hbCBKb3VybmFsIG9mIEVudmlyb25tZW50IEFncmljdWx0dXJlIGFuZCBCaW90ZWNobm9sb2d5IiwiaXNzdWUiOjIsInZvbHVtZSI6OSwiaW1wYWN0RmFjdG9yIjoiMC42NSJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjI0MiwibGFzdFBhZ2UiOjI1M319LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJJbmZvZ2FpbiBQdWJsaWNhdGlvbiJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NCwiY2l0ZWRCeUNvdW50Ijo4fX0sInF1b3RlIjoiSUlJLiBCRU5FRklUUyBPRiBQUkVDSVNJT04gSVJSSUdBVElPTiAxLiBJbXByb3ZlZCBXYXRlciBVc2UgRWZmaWNpZW5jeTogUHJlY2lzaW9uIGlycmlnYXRpb24gb3B0aW1pemVzIHdhdGVyIHVzZSBlZmZpY2llbmN5IGJ5IGRlbGl2ZXJpbmcgd2F0ZXIgZGlyZWN0bHkgdG8gdGhlIHJvb3Qgem9uZSBvZiBjcm9wcywgbWluaW1pemluZyBsb3NzZXMgIGR1ZSB0byBldmFwb3JhdGlvbiwgcnVub2ZmLCBhbmQgZGVlcCBwZXJjb2xhdGlvbi4gQnkgIG1hdGNoaW5nIHdhdGVyIGFwcGxpY2F0aW9uIHJhdGVzIHRvIGNyb3Agd2F0ZXIgIHJlcXVpcmVtZW50cywgcHJlY2lzaW9uIGlycmlnYXRpb24gc3lzdGVtcyBlbnN1cmUgdGhhdCAgd2F0ZXIgaXMgdXNlZCBtb3JlIGVmZmljaWVudGx5LCBtYXhpbWl6aW5nIGNyb3AgeWllbGRzICBwZXIgdW5pdCBvZiB3YXRlciBhcHBsaWVkLiAyLiBFbmhhbmNlZCBDcm9wIFBlcmZvcm1hbmNlOiBCeSBwcm92aWRpbmcgY3JvcHMgd2l0aCBvcHRpbWFsIG1vaXN0dXJlIGxldmVscywgcHJlY2lzaW9uIGlycmlnYXRpb24gc3lzdGVtcyBwcm9tb3RlIGhlYWx0aHkgcm9vdCBkZXZlbG9wbWVudCwgYmFsYW5jZWQgZ3Jvd3RoLCAgYW5kIGltcHJvdmVkIGNyb3Agdmlnb3IuIENvbnNpc3RlbnQgc29pbCBtb2lzdHVyZSAgbGV2ZWxzIGhlbHAgcmVkdWNlIHdhdGVyIHN0cmVzcywgaW5jcmVhc2UgbnV0cmllbnQgIHVwdGFrZSwgYW5kIGVuaGFuY2UgY3JvcCByZXNpbGllbmNlIHRvIGVudmlyb25tZW50YWwgIHN0cmVzc2VzIHN1Y2ggYXMgZHJvdWdodCBhbmQgaGVhdC4gMy4gQ29zdCBTYXZpbmdzOiBQcmVjaXNpb24gaXJyaWdhdGlvbiBzeXN0ZW1zIGhlbHAgZmFybWVycyByZWR1Y2Ugd2F0ZXIsIGVuZXJneSwgYW5kIGxhYm9yIGNvc3RzIGFzc29jaWF0ZWQgd2l0aCBpcnJpZ2F0aW9uLiBCeSBvcHRpbWl6aW5nIHdhdGVyIHVzZSBlZmZpY2llbmN5IGFuZCAgbWluaW1pemluZyBpbnB1dCB3YXN0YWdlLCBwcmVjaXNpb24gaXJyaWdhdGlvbiBzeXN0ZW1zICBjYW4gbGVhZCB0byBzaWduaWZpY2FudCBjb3N0IHNhdmluZ3Mgb3ZlciB0aW1lLCAgaW1wcm92aW5nIHRoZSBwcm9maXRhYmlsaXR5IG9mIGFncmljdWx0dXJhbCBvcGVyYXRpb25zLiA0LiBFbnZpcm9ubWVudGFsIFN1c3RhaW5hYmlsaXR5OiAgUHJlY2lzaW9uICBpcnJpZ2F0aW9uIGNvbnRyaWJ1dGVzIHRvICBlbnZpcm9ubWVudGFsICBzdXN0YWluYWJpbGl0eSAgYnkgY29uc2VydmluZyB3YXRlciAgcmVzb3VyY2VzLCAgbWluaW1pemluZyAgd2F0ZXIgcG9sbHV0aW9uLCBhbmQgcmVkdWNpbmcgZ3JlZW5ob3VzZSBnYXMgZW1pc3Npb25zLiBCeSBhZG9wdGluZyBwcmVjaXNpb24gaXJyaWdhdGlvbiBwcmFjdGljZXMsIGZhcm1lcnMgY2FuICBtaW5pbWl6ZSB0aGVpciBlbnZpcm9ubWVudGFsIGZvb3RwcmludCBhbmQgY29udHJpYnV0ZSB0byB0aGUgIGxvbmctdGVybSAgc3VzdGFpbmFiaWxpdHkgIG9mICBhZ3JpY3VsdHVyYWwgcHJvZHVjdGlvbiBzeXN0ZW1zLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjQ0Ljg2NTY0MjU0NzYwNzQyLCJsZWZ0Ijo1My40MDgyNTI3MTYwNjQ0NSwid2lkdGgiOjM4LjI2MTc2ODM0MTA2NDQ1LCJoZWlnaHQiOjEuNjY5MjI3MzYxNjc5MDc3MX19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjQ2Ljg5NzkyMjUxNTg2OTE0LCJsZWZ0Ijo1Mi40MjA1NDM2NzA2NTQzLCJ3aWR0aCI6NDAuMjQwNzM3OTE1MDM5MDYsImhlaWdodCI6MS42MjU0NTQ3ODM0Mzk2MzYyfX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NDguNDkxNzkwNzcxNDg0Mzc1LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3Ljk4MDI3ODAxNTEzNjcyLCJoZWlnaHQiOjExLjAxMzcxMzgzNjY2OTkyMn19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjU5LjkzOTU0MDg2MzAzNzExLCJsZWZ0Ijo1Mi40MjA1NDM2NzA2NTQzLCJ3aWR0aCI6NDAuMjMwMjI0NjA5Mzc1LCJoZWlnaHQiOjEuNjI1NDU0NzgzNDM5NjM2Mn19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjYxLjUzMzQxNjc0ODA0Njg3NSwibGVmdCI6NTQuNjk4MzEwODUyMDUwNzgsIndpZHRoIjozNy45NjMxMjMzMjE1MzMyLCJoZWlnaHQiOjkuNDM1MTcwMTczNjQ1MDJ9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo3MS40MTY4Nzc3NDY1ODIwMywibGVmdCI6NTIuNDIwNTQzNjcwNjU0Mywid2lkdGgiOjQwLjIxMzMxMDI0MTY5OTIyLCJoZWlnaHQiOjEuNjI1NDU4MzU5NzE4MzIyOH19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjczLjAxMDc1NzQ0NjI4OTA2LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3Ljk0NTc4NTUyMjQ2MDk0LCJoZWlnaHQiOjcuODYzNzY0Mjg2MDQxMjZ9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo4MS4zMDg1NTU2MDMwMjczNCwibGVmdCI6NTIuNDIwNTQzNjcwNjU0Mywid2lkdGgiOjQwLjI0MDM3NTUxODc5ODgzLCJoZWlnaHQiOjEuNjI1NDU4MzU5NzE4MzIyOH19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjgyLjkxOTA1OTc1MzQxNzk3LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3LjkzNjQxNjYyNTk3NjU2LCJoZWlnaHQiOjEuNTc4MTMzNTgzMDY4ODQ3N319LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjg0LjQ4NjkwMDMyOTU4OTg0LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3LjkzNDc1NzIzMjY2NjAxNiwiaGVpZ2h0IjoxLjU3ODEzMzU4MzA2ODg0Nzd9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo4Ni4wNTQ3NDg1MzUxNTYyNSwibGVmdCI6NTQuNjk4MzEwODUyMDUwNzgsIndpZHRoIjozNy45Nzg0MzE3MDE2NjAxNTYsImhlaWdodCI6NC43MTM4MTkwMjY5NDcwMjE1fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6OC40MjAyOTI4NTQzMDkwODIsImxlZnQiOjEwLjAyMjE3MjkyNzg1NjQ0NSwid2lkdGgiOjM3Ljk0NjgzMDc0OTUxMTcyLCJoZWlnaHQiOjEuNTc4MTM1MjUxOTk4OTAxNH19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjkuOTg4MTMyNDc2ODA2NjQsImxlZnQiOjEwLjAyMjE3MjkyNzg1NjQ0NSwid2lkdGgiOjEzLjg2MjQ2MTA5MDA4Nzg5LCJoZWlnaHQiOjEuNTc4MTM1MjUxOTk4OTAxNH19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjI0OSJ9fV0%3D.

### Crop Selection and Drought-Resistant Varieties

Strategic crop selection and breeding drought-resistant varieties reduce water demands inherently, promoting sustainable intensification by matching crops to local conditions and climate projections (Daraz et al., 2025)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlVtYXIiLCJmYW1pbHkiOiJEYXJheiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiLFoHRlZmFuIiwiZmFtaWx5IjoiQm9qbmVjIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllvdW5hcyIsImZhbWlseSI6IktoYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMDc3LTA0NzIiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzOTAvYWdyaWN1bHR1cmUxNTA1MDU1NCIsIm9wZW5hbGV4IjoiVzQ0MDgyMTk4MTEiLCJleGEiOiJodHRwczovL3d3dy5tZHBpLmNvbS8yMDc3LTA0NzIvMTUvNS81NTQifSwidGl0bGUiOiJFbmVyZ3ktRWZmaWNpZW50IFNtYXJ0IElycmlnYXRpb24gVGVjaG5vbG9naWVzOiBBIFBhdGh3YXkgdG8gV2F0ZXIgYW5kIEVuZXJneSBTdXN0YWluYWJpbGl0eSBpbiBBZ3JpY3VsdHVyZSIsImFic3RyYWN0IjoiVGhlIGFncmljdWx0dXJhbCBzZWN0b3IgZmFjZXMgY2hhbGxlbmdlcyBzdWNoIGFzIHdhdGVyIHNjYXJjaXR5LCBlbmVyZ3kgaW5lZmZpY2llbmN5LCBhbmQgZGVjbGluaW5nIHByb2R1Y3Rpdml0eSwgcGFydGljdWxhcmx5IGluIGFyaWQgcmVnaW9ucy4gVHJhZGl0aW9uYWwgaXJyaWdhdGlvbiBtZXRob2RzIGNvbnRyaWJ1dGUgdG8gcmVzb3VyY2UgZGVwbGV0aW9uIGFuZCBlbnZpcm9ubWVudGFsIGltcGFjdHMuIFNvbGFyLXBvd2VyZWQgc21hcnQgaXJyaWdhdGlvbiBzeXN0ZW1zIGludGVncmF0ZSBwcmVjaXNpb24gaXJyaWdhdGlvbiB3aXRoIHJlbmV3YWJsZSBlbmVyZ3ksIGltcHJvdmluZyB3YXRlciB1c2UgYW5kIHByb2R1Y3Rpdml0eS4gSW4gUGFraXN0YW4sIHdoZXJlIGFncmljdWx0dXJlIGNvbnRyaWJ1dGVzIDE5JSBvZiBncm9zcyBkb21lc3RpYyBwcm9kdWN0IGFuZCBlbXBsb3lzIDQwJSBvZiB0aGUgd29ya2ZvcmNlLCB0aGVzZSBjaGFsbGVuZ2VzIGFyZSBzZXZlcmUsIGVzcGVjaWFsbHkgaW4gd2F0ZXItc2NhcmNlIGFyZWFzIGxpa2UgdGhlIENob2xpc3RhbiBEZXNlcnQuIFRoaXMgc3R1ZHkgZXhhbWluZXMgdGhlIGltcGFjdCBvZiBzb2xhci1wb3dlcmVkIHNtYXJ0IGlycmlnYXRpb24gb24gYWdyaWN1bHR1cmFsIHByb2R1Y3Rpdml0eSwgd2F0ZXIgY29uc2VydmF0aW9uLCBhbmQgZW5lcmd5IGVmZmljaWVuY3kgaW4gdGhlIENob2xpc3RhbiBEZXNlcnQuIFVzaW5nIGEgcXVhbnRpdGF0aXZlIGNyb3NzLXNlY3Rpb25hbCBkZXNpZ24sIGRhdGEgd2VyZSBjb2xsZWN0ZWQgZnJvbSAzODQgZmFybWVycyB2aWEgc3RydWN0dXJlZCBxdWVzdGlvbm5haXJlcy4gU3RhdGlzdGljYWwgYW5hbHlzZXMsIGluY2x1ZGluZyBtdWx0aXBsZSBsaW5lYXIgcmVncmVzc2lvbiwgcGFpcmVkIHNhbXBsZSB0LXRlc3RzLCBhbmQgU3RydWN0dXJhbCBFcXVhdGlvbiBNb2RlbGluZyAoU0VNKSwgd2VyZSBjb25kdWN0ZWQuIEZpbmRpbmdzIHNob3cgc2lnbmlmaWNhbnQgaW1wcm92ZW1lbnRzIGluIGNyb3AgeWllbGQgKGZyb20gMy4wIHRvIDQuOCB0b25zL2hlY3RhcmUpIGFuZCByZWR1Y3Rpb25zIGluIHdhdGVyIGFuZCBlbmVyZ3kgY29uc3VtcHRpb24uIFJlZ3Jlc3Npb24gYW5hbHlzaXMgaGlnaGxpZ2h0ZWQgc3Ryb25nIHBvc2l0aXZlIGVmZmVjdHMgb24geWllbGQgYW5kIGVmZmljaWVuY3ksIHdoaWxlIFNFTSBjb25maXJtZWQgcmVkdWNlZCBlbnZpcm9ubWVudGFsIGltcGFjdCBhbmQgb3BlcmF0aW9uYWwgY29zdHMuIFRoZSBzdHVkeSBjb25jbHVkZXMgdGhhdCBzb2xhci1wb3dlcmVkIGlycmlnYXRpb24gZW5oYW5jZXMgcHJvZHVjdGl2aXR5LCBjb25zZXJ2ZXMgcmVzb3VyY2VzLCBhbmQgcHJvbW90ZXMgc3VzdGFpbmFiaWxpdHkuIFBvbGljeW1ha2VycyBzaG91bGQgcHJvdmlkZSBmaW5hbmNpYWwgaW5jZW50aXZlcywgaW52ZXN0IGluIHJlbmV3YWJsZSBpbmZyYXN0cnVjdHVyZSwgYW5kIGltcGxlbWVudCB0cmFpbmluZyBwcm9ncmFtcyB0byBzdXBwb3J0IGFkb3B0aW9uLiBDb2xsYWJvcmF0aXZlIGVmZm9ydHMgYXJlIGVzc2VudGlhbCBmb3Igc3VzdGFpbmFibGUgYWdyaWN1bHR1cmUgaW4gYXJpZCByZWdpb25zLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJpc3N1ZWREYXRlIjoiMjAyNS0wMy0wNSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkFncmljdWx0dXJlIiwiaXNzdWUiOjUsInZvbHVtZSI6MTUsImltcGFjdEZhY3RvciI6IjUuMjYifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjo1NTQsImxhc3RQYWdlIjo1NTR9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiTXVsdGlkaXNjaXBsaW5hcnkgRGlnaXRhbCBQdWJsaXNoaW5nIEluc3RpdHV0ZSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NDgsImNpdGVkQnlDb3VudCI6Mn19LCJxdW90ZSI6IjEuIEludHJvZHVjdGlvbiBUaGUgYWdyaWN1bHR1cmFsIHNlY3RvciwgYSBjb3JuZXJzdG9uZSBvZiBnbG9iYWwgZm9vZCBzZWN1cml0eSBhbmQgZWNvbm9taWMgZGV2ZWxvcG1lbnQsIGZhY2VzIG1vdW50aW5nIGNoYWxsZW5nZXMgaW4gdGhlIDIxc3QgY2VudHVyeS4gQ2xpbWF0ZSBjaGFuZ2UsIHJhcGlkIHBvcHVsYXRpb24gZ3Jvd3RoLCBhbmQgZHdpbmRsaW5nIG5hdHVyYWwgcmVzb3VyY2VzIGhhdmUgaW50ZW5zaWZpZWQgdGhlIG5lZWQgZm9yIHN1c3RhaW5hYmxlIGFncmljdWx0dXJhbCBwcmFjdGljZXMuIEFtb25nIHRoZXNlIGNoYWxsZW5nZXMsIHdhdGVyIHNjYXJjaXR5IGFuZCBlbmVyZ3kgaW5lZmZpY2llbmN5IHN0YW5kIG91dCBhcyBjcml0aWNhbCBiYXJyaWVycyB0byBwcm9kdWN0aXZpdHkgYW5kIHN1c3RhaW5hYmlsaXR5LiBHbG9iYWxseSwgYWdyaWN1bHR1cmUgYWNjb3VudHMgZm9yIGFwcHJveGltYXRlbHkgNzAlIG9mIGZyZXNod2F0ZXIgd2l0aGRyYXdhbHMgYW5kIGlzIG9uZSBvZiB0aGUgbGFyZ2VzdCBjb25zdW1lcnMgb2YgZW5lcmd5IGluIG1hbnkgcmVnaW9ucyBbIDEsIDJdLiBUcmFkaXRpb25hbCBpcnJpZ2F0aW9uIHByYWN0aWNlcywgb2Z0ZW4gaW5lZmZpY2llbnQgYW5kIHJlc291cmNlLWludGVuc2l2ZSwgZXhhY2VyYmF0ZSB0aGVzZSBpc3N1ZXMsIGxlYWRpbmcgdG8gd2F0ZXIgd2FzdGFnZSwgZW5lcmd5IG92ZXJ1c2UsIGFuZCByZWR1Y2VkIGNyb3AgeWllbGRzLiJ9XQ%3D%3D.

### Table 9: Types of Wastewater for Reuse

| Source Type | Treatment Required | Best Agricultural Application |
| --- | --- | --- |
| Greywater | Low to Moderate | Ornamental plants/specific crops |
| Treated Sewage | High | Forage crops/non-food items |
| Industrial Effluent | Very High | Controlled industrial farming |
| Runoff Capture | Moderate | Supplementing main sources |
| Saline Water | Desalination | Arid region crop production |
| Reclaimed Water | High | General irrigation (non-potable) |
| Agricultural Runoff | Minimal/Moderate | Closed-loop field systems |
| Stormwater | Moderate | Landscaping and soil recharge |

Principles of Operation

Genomic selection and marker-assisted breeding develop varieties with traits like deep roots, efficient stomata, and osmotic adjustment; rotated with cover crops for soil health.

Advantages and Efficiency Gains

- Intrinsic WUE: 15-30% less water for same yields (Kushwaha et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjMwLCJzdWJ0eXBlIjp7fSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTWFuaXNoIiwiZmFtaWx5IjoiS3VzaHdhaGEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2hhbmthciIsImZhbWlseSI6IlNpbmdoIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlZpamF5IEthbnQiLCJmYW1pbHkiOiJTaW5naCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTaGFzaGFuayIsImZhbWlseSI6IkR3aXZlZGkifX1dLCJpZHMiOnsiaXNzbiI6WyIyNDU2LTE4NzgiXSwiaXNibiI6W10sImRvaSI6IjEwLjIyMTYxL2lqZWFiLjkyLjI3Iiwib3BlbmFsZXgiOiJXNDM5NjU2MzU4NCJ9LCJ0aXRsZSI6IlByZWNpc2lvbiBGYXJtaW5nOiBBIFJldmlldyBvZiBNZXRob2RzLCBUZWNobm9sb2dpZXMsIGFuZCBGdXR1cmUgUHJvc3BlY3RzIiwiYWJzdHJhY3QiOiJQcmVjaXNpb24gZmFybWluZywgb2Z0ZW4gcmVmZXJyZWQgdG8gYXMgcHJlY2lzaW9uIGFncmljdWx0dXJlLCBpcyBhIGZhcm1pbmcgbWFuYWdlbWVudCBjb25jZXB0IHRoYXQgc2Vla3MgdG8gbWF4aW1pemUgYWdyaWN1bHR1cmFsIG9wZXJhdGlvbnMgdmlhIHRoZSB1c2Ugb2YgdGVjaG5vbG9neSBpbiBvcmRlciB0byBhY2hpZXZlIGdyZWF0ZXIgbGV2ZWxzIG9mIGVmZmljaWVuY3ksIHByb2R1Y3Rpb24sIGFuZCBjb25zZXJ2YXRpb24gb2YgcmVzb3VyY2VzLiBUaGUgcHVycG9zZSBvZiB0aGlzIHN1cnZleSBzdHVkeSBpcyB0byBvZmZlciBhbiBvdmVydmlldyBvZiBwcmVjaXNpb24gZmFybWluZywgaW5jbHVkaW5nIHRvcGljcyBzdWNoIGFzIGl0cyBmdW5kYW1lbnRhbHMsIG1ldGhvZG9sb2dpZXMsIHRlY2hub2xvZ3ksIGFuZCBhcHBsaWNhdGlvbnMuIEl0IGludmVzdGlnYXRlcyB0aGUgcm9sZSB0aGF0IGEgdmFyaWV0eSBvZiB0ZWNobm9sb2dpZXMsIGluY2x1ZGluZyBnbG9iYWwgbmF2aWdhdGlvbiBzYXRlbGxpdGUgc3lzdGVtcyAoR05TUyksIGdlb2dyYXBoaWMgaW5mb3JtYXRpb24gc3lzdGVtcyAoR0lTKSwgcmVtb3RlIHNlbnNpbmcsIGFuZCBzZW5zb3IgdGVjaG5vbG9neSwgcGxheSBpbiBtYWtpbmcgaXQgcG9zc2libGUgdG8gaW1wbGVtZW50IHByZWNpc2lvbiBhZ3JpY3VsdHVyYWwgbWV0aG9kcy4gSW4gYWRkaXRpb24sIGl0IGFkZHJlc3NlcyB0aGUgYWR2YW50YWdlcywgZGlzYWR2YW50YWdlcywgYW5kIHBvdGVudGlhbCBmdXR1cmUgYXBwbGljYXRpb25zIG9mIHByZWNpc2lvbiBmYXJtaW5nIGluIHRoZSBjb250ZXh0IG9mIHNvbHZpbmcgaXNzdWVzIHJlbGF0ZWQgdG8gdGhlIHN1c3RhaW5hYmlsaXR5IG9mIGFncmljdWx0dXJlIGFuZCB0aGUgc2VjdXJpdHkgb2YgZm9vZCBzdXBwbGllcyBvbiBhIHdvcmxkd2lkZSBzY2FsZS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjIxNjEvaWplYWIuOTIuMjciLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vaWplYWIuY29tL3VwbG9hZF9kb2N1bWVudC9pc3N1ZV9maWxlcy8yN0lKRUFCLTEwNDIwMjQ4NC1QcmVjaXNpb25GYXJtaW5nLnBkZiIsImlzc3VlZERhdGUiOiIyMDI0LTAxLTAxIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiSW50ZXJuYXRpb25hbCBKb3VybmFsIG9mIEVudmlyb25tZW50IEFncmljdWx0dXJlIGFuZCBCaW90ZWNobm9sb2d5IiwiaXNzdWUiOjIsInZvbHVtZSI6OSwiaW1wYWN0RmFjdG9yIjoiMC42NSJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjI0MiwibGFzdFBhZ2UiOjI1M319LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJJbmZvZ2FpbiBQdWJsaWNhdGlvbiJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NCwiY2l0ZWRCeUNvdW50Ijo4fX0sInF1b3RlIjoiSUlJLiBCRU5FRklUUyBPRiBQUkVDSVNJT04gSVJSSUdBVElPTiAxLiBJbXByb3ZlZCBXYXRlciBVc2UgRWZmaWNpZW5jeTogUHJlY2lzaW9uIGlycmlnYXRpb24gb3B0aW1pemVzIHdhdGVyIHVzZSBlZmZpY2llbmN5IGJ5IGRlbGl2ZXJpbmcgd2F0ZXIgZGlyZWN0bHkgdG8gdGhlIHJvb3Qgem9uZSBvZiBjcm9wcywgbWluaW1pemluZyBsb3NzZXMgIGR1ZSB0byBldmFwb3JhdGlvbiwgcnVub2ZmLCBhbmQgZGVlcCBwZXJjb2xhdGlvbi4gQnkgIG1hdGNoaW5nIHdhdGVyIGFwcGxpY2F0aW9uIHJhdGVzIHRvIGNyb3Agd2F0ZXIgIHJlcXVpcmVtZW50cywgcHJlY2lzaW9uIGlycmlnYXRpb24gc3lzdGVtcyBlbnN1cmUgdGhhdCAgd2F0ZXIgaXMgdXNlZCBtb3JlIGVmZmljaWVudGx5LCBtYXhpbWl6aW5nIGNyb3AgeWllbGRzICBwZXIgdW5pdCBvZiB3YXRlciBhcHBsaWVkLiAyLiBFbmhhbmNlZCBDcm9wIFBlcmZvcm1hbmNlOiBCeSBwcm92aWRpbmcgY3JvcHMgd2l0aCBvcHRpbWFsIG1vaXN0dXJlIGxldmVscywgcHJlY2lzaW9uIGlycmlnYXRpb24gc3lzdGVtcyBwcm9tb3RlIGhlYWx0aHkgcm9vdCBkZXZlbG9wbWVudCwgYmFsYW5jZWQgZ3Jvd3RoLCAgYW5kIGltcHJvdmVkIGNyb3Agdmlnb3IuIENvbnNpc3RlbnQgc29pbCBtb2lzdHVyZSAgbGV2ZWxzIGhlbHAgcmVkdWNlIHdhdGVyIHN0cmVzcywgaW5jcmVhc2UgbnV0cmllbnQgIHVwdGFrZSwgYW5kIGVuaGFuY2UgY3JvcCByZXNpbGllbmNlIHRvIGVudmlyb25tZW50YWwgIHN0cmVzc2VzIHN1Y2ggYXMgZHJvdWdodCBhbmQgaGVhdC4gMy4gQ29zdCBTYXZpbmdzOiBQcmVjaXNpb24gaXJyaWdhdGlvbiBzeXN0ZW1zIGhlbHAgZmFybWVycyByZWR1Y2Ugd2F0ZXIsIGVuZXJneSwgYW5kIGxhYm9yIGNvc3RzIGFzc29jaWF0ZWQgd2l0aCBpcnJpZ2F0aW9uLiBCeSBvcHRpbWl6aW5nIHdhdGVyIHVzZSBlZmZpY2llbmN5IGFuZCAgbWluaW1pemluZyBpbnB1dCB3YXN0YWdlLCBwcmVjaXNpb24gaXJyaWdhdGlvbiBzeXN0ZW1zICBjYW4gbGVhZCB0byBzaWduaWZpY2FudCBjb3N0IHNhdmluZ3Mgb3ZlciB0aW1lLCAgaW1wcm92aW5nIHRoZSBwcm9maXRhYmlsaXR5IG9mIGFncmljdWx0dXJhbCBvcGVyYXRpb25zLiA0LiBFbnZpcm9ubWVudGFsIFN1c3RhaW5hYmlsaXR5OiAgUHJlY2lzaW9uICBpcnJpZ2F0aW9uIGNvbnRyaWJ1dGVzIHRvICBlbnZpcm9ubWVudGFsICBzdXN0YWluYWJpbGl0eSAgYnkgY29uc2VydmluZyB3YXRlciAgcmVzb3VyY2VzLCAgbWluaW1pemluZyAgd2F0ZXIgcG9sbHV0aW9uLCBhbmQgcmVkdWNpbmcgZ3JlZW5ob3VzZSBnYXMgZW1pc3Npb25zLiBCeSBhZG9wdGluZyBwcmVjaXNpb24gaXJyaWdhdGlvbiBwcmFjdGljZXMsIGZhcm1lcnMgY2FuICBtaW5pbWl6ZSB0aGVpciBlbnZpcm9ubWVudGFsIGZvb3RwcmludCBhbmQgY29udHJpYnV0ZSB0byB0aGUgIGxvbmctdGVybSAgc3VzdGFpbmFiaWxpdHkgIG9mICBhZ3JpY3VsdHVyYWwgcHJvZHVjdGlvbiBzeXN0ZW1zLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjQ0Ljg2NTY0MjU0NzYwNzQyLCJsZWZ0Ijo1My40MDgyNTI3MTYwNjQ0NSwid2lkdGgiOjM4LjI2MTc2ODM0MTA2NDQ1LCJoZWlnaHQiOjEuNjY5MjI3MzYxNjc5MDc3MX19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjQ2Ljg5NzkyMjUxNTg2OTE0LCJsZWZ0Ijo1Mi40MjA1NDM2NzA2NTQzLCJ3aWR0aCI6NDAuMjQwNzM3OTE1MDM5MDYsImhlaWdodCI6MS42MjU0NTQ3ODM0Mzk2MzYyfX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NDguNDkxNzkwNzcxNDg0Mzc1LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3Ljk4MDI3ODAxNTEzNjcyLCJoZWlnaHQiOjExLjAxMzcxMzgzNjY2OTkyMn19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjU5LjkzOTU0MDg2MzAzNzExLCJsZWZ0Ijo1Mi40MjA1NDM2NzA2NTQzLCJ3aWR0aCI6NDAuMjMwMjI0NjA5Mzc1LCJoZWlnaHQiOjEuNjI1NDU0NzgzNDM5NjM2Mn19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjYxLjUzMzQxNjc0ODA0Njg3NSwibGVmdCI6NTQuNjk4MzEwODUyMDUwNzgsIndpZHRoIjozNy45NjMxMjMzMjE1MzMyLCJoZWlnaHQiOjkuNDM1MTcwMTczNjQ1MDJ9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo3MS40MTY4Nzc3NDY1ODIwMywibGVmdCI6NTIuNDIwNTQzNjcwNjU0Mywid2lkdGgiOjQwLjIxMzMxMDI0MTY5OTIyLCJoZWlnaHQiOjEuNjI1NDU4MzU5NzE4MzIyOH19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjczLjAxMDc1NzQ0NjI4OTA2LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3Ljk0NTc4NTUyMjQ2MDk0LCJoZWlnaHQiOjcuODYzNzY0Mjg2MDQxMjZ9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo4MS4zMDg1NTU2MDMwMjczNCwibGVmdCI6NTIuNDIwNTQzNjcwNjU0Mywid2lkdGgiOjQwLjI0MDM3NTUxODc5ODgzLCJoZWlnaHQiOjEuNjI1NDU4MzU5NzE4MzIyOH19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjgyLjkxOTA1OTc1MzQxNzk3LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3LjkzNjQxNjYyNTk3NjU2LCJoZWlnaHQiOjEuNTc4MTMzNTgzMDY4ODQ3N319LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjg0LjQ4NjkwMDMyOTU4OTg0LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3LjkzNDc1NzIzMjY2NjAxNiwiaGVpZ2h0IjoxLjU3ODEzMzU4MzA2ODg0Nzd9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo4Ni4wNTQ3NDg1MzUxNTYyNSwibGVmdCI6NTQuNjk4MzEwODUyMDUwNzgsIndpZHRoIjozNy45Nzg0MzE3MDE2NjAxNTYsImhlaWdodCI6NC43MTM4MTkwMjY5NDcwMjE1fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6OC40MjAyOTI4NTQzMDkwODIsImxlZnQiOjEwLjAyMjE3MjkyNzg1NjQ0NSwid2lkdGgiOjM3Ljk0NjgzMDc0OTUxMTcyLCJoZWlnaHQiOjEuNTc4MTM1MjUxOTk4OTAxNH19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjkuOTg4MTMyNDc2ODA2NjQsImxlZnQiOjEwLjAyMjE3MjkyNzg1NjQ0NSwid2lkdGgiOjEzLjg2MjQ2MTA5MDA4Nzg5LCJoZWlnaHQiOjEuNTc4MTM1MjUxOTk4OTAxNH19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjI0OSJ9fV0%3D;

- Yield stability: Performs under deficits;

- Biodiversity: Diversifies farming systems.

## Socio-Economic and Environmental Impacts of Advanced Irrigation

### Water Use Efficiency and Conservation

Advanced irrigation technologies, from precision delivery to alternative sources, elevate WUE by minimizing losses, with gains of 25-50% reported, conserving agriculture's dominant freshwater footprint (Pérez‐Blanco et al., 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjMwLCJzdWJ0eXBlIjp7fSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQy4gRGlvbmlzaW8iLCJmYW1pbHkiOiJQw6lyZXrigJBCbGFuY28ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQXJ0aHVyIEhyYXN0IiwiZmFtaWx5IjoiRXNzZW5mZWxkZXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQ2hyaXMiLCJmYW1pbHkiOiJQZXJyeSJ9fV0sImlkcyI6eyJpc3NuIjpbIjE3NTAtNjgxNiIsIjE3NTAtNjgyNCJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMTA5My9yZWVwL3JlYWEwMDQiLCJtYWciOiIzMDgxMDMwOTg2Iiwib3BlbmFsZXgiOiJXMzA4MTAzMDk4NiIsImV4YSI6Imh0dHBzOi8vd3d3LmpvdXJuYWxzLnVjaGljYWdvLmVkdS9kb2kvMTAuMTA5My9yZWVwL3JlYWEwMDQifSwidGl0bGUiOiJJcnJpZ2F0aW9uIFRlY2hub2xvZ3kgYW5kIFdhdGVyIENvbnNlcnZhdGlvbjogQSBSZXZpZXcgb2YgdGhlIFRoZW9yeSBhbmQgRXZpZGVuY2UiLCJhYnN0cmFjdCI6IkZhcm1pbmcgYWNjb3VudHMgZm9yIGFwcHJveGltYXRlbHkgNzAgcGVyY2VudCBvZiBmcmVzaHdhdGVyIHdpdGhkcmF3YWxzIHdvcmxkd2lkZSwgYW5kIGl0IG9mdGVuIGNvbnN0aXR1dGVzIHRoZSBsb3dlc3QgdmFsdWUgdXNlIG9mIGZyZXNod2F0ZXIuIFdoZXJlIHdhdGVyIGlzIHNjYXJjZSwgYWR2YW5jZWQgaXJyaWdhdGlvbiB0ZWNobm9sb2dpZXMgc3VjaCBhcyBkcmlwIGFuZCBwaXBlZCBkZWxpdmVyeSBzeXN0ZW1zIGhhdmUgYmVlbiBwcm9tb3RlZCBhcyBcIndhdGVyIGNvbnNlcnZhdGlvbiB0ZWNobm9sb2dpZXNcIiAoV0NUcykgdGhhdCByZWR1Y2UgYWdyaWN1bHR1cmFsIHdhdGVyIGNvbnN1bXB0aW9uLCB0aHVzIHJlbGVhc2luZyB3YXRlciB0byBhbHRlcm5hdGl2ZSB1c2VzIChlLmcuLCB0aGUgZW52aXJvbm1lbnQpLiBUaGlzIGFydGljbGUgZXhhbWluZXMgd2hldGhlciB0aGlzIGlzIHRydWUsIGRpc2N1c3NpbmcgaG93IFdDVHMgY29udHJpYnV0ZSwgaW4gdGhlb3J5IGFuZCBwcmFjdGljZSwgdG8gd2F0ZXIgY29uc2VydmF0aW9uLiBCYXNlZCBvbiBhbiBleHRlbnNpdmUgcmV2aWV3IG9mIG1vcmUgdGhhbiAyMzAgdGhlb3JldGljYWwgYW5kIGVtcGlyaWNhbCBwYXBlcnMsIHdlIGFyZ3VlIHRoYXQgV0NUcyBzaG91bGQgbm90IGJlIHZpZXdlZCBhcyBhIHRvb2wgZm9yIGFjaGlldmluZyB3YXRlciBjb25zZXJ2YXRpb24sIGJ1dCByYXRoZXIgYXMgYSBtZWFucyBmb3Igc3RhYmlsaXppbmcgYW5kIGluY3JlYXNpbmcgYWdyaWN1bHR1cmFsIHdhdGVyIHByb2R1Y3Rpdml0eSBhbmQgZmFybWVycycgaW5jb21lIGluIHBsYWNlcyB3aGVyZSB3YXRlciBpcyBzY2FyY2UuIFdlIGNvbmNsdWRlIHRoYXQsIGlmIHRoZSB1bHRpbWF0ZSBvYmplY3RpdmUgaXMgd2F0ZXIgY29uc2VydmF0aW9uLCBpdCBpcyBlc3NlbnRpYWwgdG8gYWRvcHQgd2F0ZXIgY29uc2VydmF0aW9uIHBvbGljaWVz4oCUdGhhdCBpcywgZ292ZXJuYW5jZSBpbnN0cnVtZW50cyBhaW1lZCBhdCByZWFsbG9jYXRpbmcgYXZhaWxhYmxlIHJlc291cmNlcyBhbW9uZyB1c2VzIChlLmcuLCBmcm9tIGlycmlnYXRpb24gdG8gdGhlIGVudmlyb25tZW50KSIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDkzL3JlZXAvcmVhYTAwNCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjEwOTMvcmVlcC9yZWFhMDA0IiwiaXNzdWVkRGF0ZSI6IjIwMjAtMDUtMjEiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA5IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJSZXZpZXcgb2YgRW52aXJvbm1lbnRhbCBFY29ub21pY3MgYW5kIFBvbGljeSIsImlzc3VlIjoyLCJ2b2x1bWUiOjE0LCJpbXBhY3RGYWN0b3IiOiI1LjUzIn0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MjE2LCJsYXN0UGFnZSI6MjM5fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlVuaXZlcnNpdHkgb2YgQ2hpY2FnbyBQcmVzcyJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NTcsImNpdGVkQnlDb3VudCI6MTU0fX0sInF1b3RlIjoiSW50cm9kdWN0aW9uIENsaW1hdGUgY2hhbmdlLCBwb3B1bGF0aW9uIGdyb3d0aCwgYW5kIGVjb25vbWljIGRldmVsb3BtZW50IGluY3JlYXNlIGNvbXBldGl0aW9uIGZvciB3YXRlciBhbmQgZXhhY2VyYmF0ZSB3YXRlciBzY2FyY2l0eS0gYW5kIGRyb3VnaHQtcmVsYXRlZCBsb3NzZXMgKCBJUENDIDIwMTQpLCByZXN1bHRpbmcgaW4gdGhlIGlkZW50aWZpY2F0aW9uIG9mIHdhdGVyIGNyaXNlcyBhcyB0aGUgZ3JlYXRlc3QgZ2xvYmFsIHNvY2lldGFsIHRocmVhdCAoIFdFRiAyMDE5KS4gRmFybWluZyBjdXJyZW50bHkgYWNjb3VudHMgZm9yIHJvdWdobHkgNzAgcGVyY2VudCBvZiBmcmVzaHdhdGVyIHdpdGhkcmF3YWxzIHdvcmxkd2lkZSAoIEZBTyAyMDE5KSBhbmQgb2Z0ZW4gY29uc3RpdHV0ZXMgdGhlIGxlYXN0IHByb2R1Y3RpdmUgKGkuZS4sIGxvd2VzdCB2YWx1ZSkgdXNlIG9mIGZyZXNod2F0ZXIgcmVzb3VyY2VzICggRGFtYW5pYSBldMKgYWwuIDIwMTcpLiBJbiB0aGlzIGNvbnRleHQsIHByb3ZpZGluZyBzYWZlLCBzdGFibGUsIGFuZCBwcm9maXRhYmxlIGZvb2QgcHJvZHVjdGlvbiB3aGlsZSBtYWtpbmcgaW5jcmVtZW50YWwgd2F0ZXIgYXZhaWxhYmxlIHRvIGFsdGVybmF0aXZlIHVzZXMsIGluY2x1ZGluZyB0aGUgZW52aXJvbm1lbnQsIHJlcXVpcmVzIGVmZmljaWVuY3kgaW1wcm92ZW1lbnRzIGluIGFncmljdWx0dXJhbCB3YXRlciBtYW5hZ2VtZW50ICggVU4gMjAxNSkuIiwic2NvcmUiOjAuMjIyMjk5OTkzMDM4MTc3NSwiZXh0ZXJuYWxDYWNoZUlkIjoiNmUxMWQxMTctNGE1MC0zYWYxLWE3NGUtMzUwZGUzODgwYmJiIn1d. These technological advancements are crucial for mitigating water scarcity and ensuring long-term agricultural sustainability, particularly in regions facing increasing climatic variability (Antu et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjMwLCJzdWJ0eXBlIjp7fSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiVXR0YW0gQmlzd2FzIiwiZmFtaWx5IjoiQW50dSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNZC4gU2FpZnVsIiwiZmFtaWx5IjoiSXNsYW0ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU3VqYXQiLCJmYW1pbHkiOiJBaG1lZCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNZCIsImZhbWlseSI6IkFyaWZ1enphbWFuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlMgQyBTaXJrYXIgYW5kIE4gTiIsImZhbWlseSI6IlNhaGEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUHVqYSBSYW5pIiwiZmFtaWx5IjoiTWl0dSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBbmltZXNoIiwiZmFtaWx5IjoiU2Fya2FyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5vciBBaWRhIiwiZmFtaWx5IjoiTWFoaWRkaW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiWnVsaGlsbWkiLCJmYW1pbHkiOiJJc21haWwifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiS2hhbGlkIEEuIiwiZmFtaWx5IjoiSWJyYWhpbSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBYnViYWtyIE0uIiwiZmFtaWx5IjoiSWRyaXMifX1dLCJpZHMiOnsiaXNzbiI6WyIyMjE0LTcxNDQiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMTYvai5qd3BlLjIwMjQuMTA2MzE3Iiwib3BlbmFsZXgiOiJXNDQwMzU2MjczOSIsImV4YSI6Imh0dHBzOi8vd3d3LnNjaWVuY2VkaXJlY3QuY29tL3NjaWVuY2UvYXJ0aWNsZS9hYnMvcGlpL1MyMjE0NzE0NDI0MDE1NDk2In0sInRpdGxlIjoiRW1lcmdpbmcgdGVjaG5vbG9naWVzIGZvciBlZmZpY2llbnQgd2F0ZXIgdXNlIGluIGFncmljdWx0dXJlOiBBIHJldmlldyBvZiBjdXJyZW50IHRyZW5kcyBhbmQgZnV0dXJlIGRpcmVjdGlvbnMiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAxNi9qLmp3cGUuMjAyNC4xMDYzMTciLCJpc3N1ZWREYXRlIjoiMjAyNC0xMC0xOSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTEiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkpvdXJuYWwgb2YgV2F0ZXIgUHJvY2VzcyBFbmdpbmVlcmluZyIsInZvbHVtZSI6NjgsImltcGFjdEZhY3RvciI6IjcuMTYifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjoxMDYzMTcsImxhc3RQYWdlIjoxMDYzMTd9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRWxzZXZpZXIgQlYifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjI1OSwiY2l0ZWRCeUNvdW50Ijo4fX0sInF1b3RlIjoiRWZmaWNpZW50IHdhdGVyIHVzZSBpbiBbYWdyaWN1bHR1cmVdPHdlYl9saW5rPiBpcyB2aXRhbCBmb3IgW2Zvb2Qgc2VjdXJpdHldPHdlYl9saW5rPiwgZW52aXJvbm1lbnRhbCBwcm90ZWN0aW9uLCBhbmQgZWNvbm9taWMgdmlhYmlsaXR5LiBDb252ZW50aW9uYWwgd2F0ZXIgbWFuYWdlbWVudCBtZXRob2RzIGFyZSBpbmFkZXF1YXRlIGZvciBhZGRyZXNzaW5nIGVtZXJnaW5nIGNoYWxsZW5nZXMsIG5lY2Vzc2l0YXRpbmcgYWR2YW5jZWQgdGVjaG5vbG9naWVzIHN1Y2ggYXMgc2Vuc29ycywgYXJ0aWZpY2lhbCBpbnRlbGxpZ2VuY2UsIFtyZW1vdGUgc2Vuc2luZ108d2ViX2xpbms%2BLCBuYW5vdGVjaG5vbG9neSwgYW5kIGRlc2FsaW5hdGlvbi4iLCJleHRlcm5hbENhY2hlSWQiOiI4YmY1ZjU1OS04YTUxLTM5Y2MtYmU1YS1kNGUzNjlhNjY0ZTkifV0%3D.

### Table 10: Environmental Impact of Conventional Methods

| Impact Type | Description | Consequences |
| --- | --- | --- |
| Soil Salinization | Salt buildup from runoff | Reduced soil fertility |
| Nutrient Leaching | Nitrate loss through soil | Groundwater contamination |
| Soil Erosion | Water-driven topsoil loss | Permanent land productivity loss |
| Carbon Emissions | Fuel for water pumping | Contribution to climate change |
| Water Depletion | Aquifer over-extraction | Dry wells/resource scarcity |
| Surface Runoff | Chemical transport | Pollution of downstream waters |
| Inefficiency | Low use efficiency | Escalating pressure on resources |
| Habitats | Ecosystem disruption | Loss of local biodiversity |

### Energy Consumption and Greenhouse Gas Emissions

By optimizing application and reducing pumping/excess, these systems cut energy use and associated GHG emissions, aligning with sustainability goals (Daraz et al., 2025; Kushwaha et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlVtYXIiLCJmYW1pbHkiOiJEYXJheiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiLFoHRlZmFuIiwiZmFtaWx5IjoiQm9qbmVjIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllvdW5hcyIsImZhbWlseSI6IktoYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMDc3LTA0NzIiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzOTAvYWdyaWN1bHR1cmUxNTA1MDU1NCIsIm9wZW5hbGV4IjoiVzQ0MDgyMTk4MTEiLCJleGEiOiJodHRwczovL3d3dy5tZHBpLmNvbS8yMDc3LTA0NzIvMTUvNS81NTQifSwidGl0bGUiOiJFbmVyZ3ktRWZmaWNpZW50IFNtYXJ0IElycmlnYXRpb24gVGVjaG5vbG9naWVzOiBBIFBhdGh3YXkgdG8gV2F0ZXIgYW5kIEVuZXJneSBTdXN0YWluYWJpbGl0eSBpbiBBZ3JpY3VsdHVyZSIsImFic3RyYWN0IjoiVGhlIGFncmljdWx0dXJhbCBzZWN0b3IgZmFjZXMgY2hhbGxlbmdlcyBzdWNoIGFzIHdhdGVyIHNjYXJjaXR5LCBlbmVyZ3kgaW5lZmZpY2llbmN5LCBhbmQgZGVjbGluaW5nIHByb2R1Y3Rpdml0eSwgcGFydGljdWxhcmx5IGluIGFyaWQgcmVnaW9ucy4gVHJhZGl0aW9uYWwgaXJyaWdhdGlvbiBtZXRob2RzIGNvbnRyaWJ1dGUgdG8gcmVzb3VyY2UgZGVwbGV0aW9uIGFuZCBlbnZpcm9ubWVudGFsIGltcGFjdHMuIFNvbGFyLXBvd2VyZWQgc21hcnQgaXJyaWdhdGlvbiBzeXN0ZW1zIGludGVncmF0ZSBwcmVjaXNpb24gaXJyaWdhdGlvbiB3aXRoIHJlbmV3YWJsZSBlbmVyZ3ksIGltcHJvdmluZyB3YXRlciB1c2UgYW5kIHByb2R1Y3Rpdml0eS4gSW4gUGFraXN0YW4sIHdoZXJlIGFncmljdWx0dXJlIGNvbnRyaWJ1dGVzIDE5JSBvZiBncm9zcyBkb21lc3RpYyBwcm9kdWN0IGFuZCBlbXBsb3lzIDQwJSBvZiB0aGUgd29ya2ZvcmNlLCB0aGVzZSBjaGFsbGVuZ2VzIGFyZSBzZXZlcmUsIGVzcGVjaWFsbHkgaW4gd2F0ZXItc2NhcmNlIGFyZWFzIGxpa2UgdGhlIENob2xpc3RhbiBEZXNlcnQuIFRoaXMgc3R1ZHkgZXhhbWluZXMgdGhlIGltcGFjdCBvZiBzb2xhci1wb3dlcmVkIHNtYXJ0IGlycmlnYXRpb24gb24gYWdyaWN1bHR1cmFsIHByb2R1Y3Rpdml0eSwgd2F0ZXIgY29uc2VydmF0aW9uLCBhbmQgZW5lcmd5IGVmZmljaWVuY3kgaW4gdGhlIENob2xpc3RhbiBEZXNlcnQuIFVzaW5nIGEgcXVhbnRpdGF0aXZlIGNyb3NzLXNlY3Rpb25hbCBkZXNpZ24sIGRhdGEgd2VyZSBjb2xsZWN0ZWQgZnJvbSAzODQgZmFybWVycyB2aWEgc3RydWN0dXJlZCBxdWVzdGlvbm5haXJlcy4gU3RhdGlzdGljYWwgYW5hbHlzZXMsIGluY2x1ZGluZyBtdWx0aXBsZSBsaW5lYXIgcmVncmVzc2lvbiwgcGFpcmVkIHNhbXBsZSB0LXRlc3RzLCBhbmQgU3RydWN0dXJhbCBFcXVhdGlvbiBNb2RlbGluZyAoU0VNKSwgd2VyZSBjb25kdWN0ZWQuIEZpbmRpbmdzIHNob3cgc2lnbmlmaWNhbnQgaW1wcm92ZW1lbnRzIGluIGNyb3AgeWllbGQgKGZyb20gMy4wIHRvIDQuOCB0b25zL2hlY3RhcmUpIGFuZCByZWR1Y3Rpb25zIGluIHdhdGVyIGFuZCBlbmVyZ3kgY29uc3VtcHRpb24uIFJlZ3Jlc3Npb24gYW5hbHlzaXMgaGlnaGxpZ2h0ZWQgc3Ryb25nIHBvc2l0aXZlIGVmZmVjdHMgb24geWllbGQgYW5kIGVmZmljaWVuY3ksIHdoaWxlIFNFTSBjb25maXJtZWQgcmVkdWNlZCBlbnZpcm9ubWVudGFsIGltcGFjdCBhbmQgb3BlcmF0aW9uYWwgY29zdHMuIFRoZSBzdHVkeSBjb25jbHVkZXMgdGhhdCBzb2xhci1wb3dlcmVkIGlycmlnYXRpb24gZW5oYW5jZXMgcHJvZHVjdGl2aXR5LCBjb25zZXJ2ZXMgcmVzb3VyY2VzLCBhbmQgcHJvbW90ZXMgc3VzdGFpbmFiaWxpdHkuIFBvbGljeW1ha2VycyBzaG91bGQgcHJvdmlkZSBmaW5hbmNpYWwgaW5jZW50aXZlcywgaW52ZXN0IGluIHJlbmV3YWJsZSBpbmZyYXN0cnVjdHVyZSwgYW5kIGltcGxlbWVudCB0cmFpbmluZyBwcm9ncmFtcyB0byBzdXBwb3J0IGFkb3B0aW9uLiBDb2xsYWJvcmF0aXZlIGVmZm9ydHMgYXJlIGVzc2VudGlhbCBmb3Igc3VzdGFpbmFibGUgYWdyaWN1bHR1cmUgaW4gYXJpZCByZWdpb25zLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJpc3N1ZWREYXRlIjoiMjAyNS0wMy0wNSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkFncmljdWx0dXJlIiwiaXNzdWUiOjUsInZvbHVtZSI6MTUsImltcGFjdEZhY3RvciI6IjUuMjYifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjo1NTQsImxhc3RQYWdlIjo1NTR9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiTXVsdGlkaXNjaXBsaW5hcnkgRGlnaXRhbCBQdWJsaXNoaW5nIEluc3RpdHV0ZSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NDgsImNpdGVkQnlDb3VudCI6Mn19LCJxdW90ZSI6IjEuIEludHJvZHVjdGlvbiBUaGUgYWdyaWN1bHR1cmFsIHNlY3RvciwgYSBjb3JuZXJzdG9uZSBvZiBnbG9iYWwgZm9vZCBzZWN1cml0eSBhbmQgZWNvbm9taWMgZGV2ZWxvcG1lbnQsIGZhY2VzIG1vdW50aW5nIGNoYWxsZW5nZXMgaW4gdGhlIDIxc3QgY2VudHVyeS4gQ2xpbWF0ZSBjaGFuZ2UsIHJhcGlkIHBvcHVsYXRpb24gZ3Jvd3RoLCBhbmQgZHdpbmRsaW5nIG5hdHVyYWwgcmVzb3VyY2VzIGhhdmUgaW50ZW5zaWZpZWQgdGhlIG5lZWQgZm9yIHN1c3RhaW5hYmxlIGFncmljdWx0dXJhbCBwcmFjdGljZXMuIEFtb25nIHRoZXNlIGNoYWxsZW5nZXMsIHdhdGVyIHNjYXJjaXR5IGFuZCBlbmVyZ3kgaW5lZmZpY2llbmN5IHN0YW5kIG91dCBhcyBjcml0aWNhbCBiYXJyaWVycyB0byBwcm9kdWN0aXZpdHkgYW5kIHN1c3RhaW5hYmlsaXR5LiBHbG9iYWxseSwgYWdyaWN1bHR1cmUgYWNjb3VudHMgZm9yIGFwcHJveGltYXRlbHkgNzAlIG9mIGZyZXNod2F0ZXIgd2l0aGRyYXdhbHMgYW5kIGlzIG9uZSBvZiB0aGUgbGFyZ2VzdCBjb25zdW1lcnMgb2YgZW5lcmd5IGluIG1hbnkgcmVnaW9ucyBbIDEsIDJdLiBUcmFkaXRpb25hbCBpcnJpZ2F0aW9uIHByYWN0aWNlcywgb2Z0ZW4gaW5lZmZpY2llbnQgYW5kIHJlc291cmNlLWludGVuc2l2ZSwgZXhhY2VyYmF0ZSB0aGVzZSBpc3N1ZXMsIGxlYWRpbmcgdG8gd2F0ZXIgd2FzdGFnZSwgZW5lcmd5IG92ZXJ1c2UsIGFuZCByZWR1Y2VkIGNyb3AgeWllbGRzLiJ9LHsib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjMwLCJzdWJ0eXBlIjp7fSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTWFuaXNoIiwiZmFtaWx5IjoiS3VzaHdhaGEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2hhbmthciIsImZhbWlseSI6IlNpbmdoIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlZpamF5IEthbnQiLCJmYW1pbHkiOiJTaW5naCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTaGFzaGFuayIsImZhbWlseSI6IkR3aXZlZGkifX1dLCJpZHMiOnsiaXNzbiI6WyIyNDU2LTE4NzgiXSwiaXNibiI6W10sImRvaSI6IjEwLjIyMTYxL2lqZWFiLjkyLjI3Iiwib3BlbmFsZXgiOiJXNDM5NjU2MzU4NCJ9LCJ0aXRsZSI6IlByZWNpc2lvbiBGYXJtaW5nOiBBIFJldmlldyBvZiBNZXRob2RzLCBUZWNobm9sb2dpZXMsIGFuZCBGdXR1cmUgUHJvc3BlY3RzIiwiYWJzdHJhY3QiOiJQcmVjaXNpb24gZmFybWluZywgb2Z0ZW4gcmVmZXJyZWQgdG8gYXMgcHJlY2lzaW9uIGFncmljdWx0dXJlLCBpcyBhIGZhcm1pbmcgbWFuYWdlbWVudCBjb25jZXB0IHRoYXQgc2Vla3MgdG8gbWF4aW1pemUgYWdyaWN1bHR1cmFsIG9wZXJhdGlvbnMgdmlhIHRoZSB1c2Ugb2YgdGVjaG5vbG9neSBpbiBvcmRlciB0byBhY2hpZXZlIGdyZWF0ZXIgbGV2ZWxzIG9mIGVmZmljaWVuY3ksIHByb2R1Y3Rpb24sIGFuZCBjb25zZXJ2YXRpb24gb2YgcmVzb3VyY2VzLiBUaGUgcHVycG9zZSBvZiB0aGlzIHN1cnZleSBzdHVkeSBpcyB0byBvZmZlciBhbiBvdmVydmlldyBvZiBwcmVjaXNpb24gZmFybWluZywgaW5jbHVkaW5nIHRvcGljcyBzdWNoIGFzIGl0cyBmdW5kYW1lbnRhbHMsIG1ldGhvZG9sb2dpZXMsIHRlY2hub2xvZ3ksIGFuZCBhcHBsaWNhdGlvbnMuIEl0IGludmVzdGlnYXRlcyB0aGUgcm9sZSB0aGF0IGEgdmFyaWV0eSBvZiB0ZWNobm9sb2dpZXMsIGluY2x1ZGluZyBnbG9iYWwgbmF2aWdhdGlvbiBzYXRlbGxpdGUgc3lzdGVtcyAoR05TUyksIGdlb2dyYXBoaWMgaW5mb3JtYXRpb24gc3lzdGVtcyAoR0lTKSwgcmVtb3RlIHNlbnNpbmcsIGFuZCBzZW5zb3IgdGVjaG5vbG9neSwgcGxheSBpbiBtYWtpbmcgaXQgcG9zc2libGUgdG8gaW1wbGVtZW50IHByZWNpc2lvbiBhZ3JpY3VsdHVyYWwgbWV0aG9kcy4gSW4gYWRkaXRpb24sIGl0IGFkZHJlc3NlcyB0aGUgYWR2YW50YWdlcywgZGlzYWR2YW50YWdlcywgYW5kIHBvdGVudGlhbCBmdXR1cmUgYXBwbGljYXRpb25zIG9mIHByZWNpc2lvbiBmYXJtaW5nIGluIHRoZSBjb250ZXh0IG9mIHNvbHZpbmcgaXNzdWVzIHJlbGF0ZWQgdG8gdGhlIHN1c3RhaW5hYmlsaXR5IG9mIGFncmljdWx0dXJlIGFuZCB0aGUgc2VjdXJpdHkgb2YgZm9vZCBzdXBwbGllcyBvbiBhIHdvcmxkd2lkZSBzY2FsZS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjIxNjEvaWplYWIuOTIuMjciLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vaWplYWIuY29tL3VwbG9hZF9kb2N1bWVudC9pc3N1ZV9maWxlcy8yN0lKRUFCLTEwNDIwMjQ4NC1QcmVjaXNpb25GYXJtaW5nLnBkZiIsImlzc3VlZERhdGUiOiIyMDI0LTAxLTAxIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiSW50ZXJuYXRpb25hbCBKb3VybmFsIG9mIEVudmlyb25tZW50IEFncmljdWx0dXJlIGFuZCBCaW90ZWNobm9sb2d5IiwiaXNzdWUiOjIsInZvbHVtZSI6OSwiaW1wYWN0RmFjdG9yIjoiMC42NSJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjI0MiwibGFzdFBhZ2UiOjI1M319LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJJbmZvZ2FpbiBQdWJsaWNhdGlvbiJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NCwiY2l0ZWRCeUNvdW50Ijo4fX0sInF1b3RlIjoiSUlJLiBCRU5FRklUUyBPRiBQUkVDSVNJT04gSVJSSUdBVElPTiAxLiBJbXByb3ZlZCBXYXRlciBVc2UgRWZmaWNpZW5jeTogUHJlY2lzaW9uIGlycmlnYXRpb24gb3B0aW1pemVzIHdhdGVyIHVzZSBlZmZpY2llbmN5IGJ5IGRlbGl2ZXJpbmcgd2F0ZXIgZGlyZWN0bHkgdG8gdGhlIHJvb3Qgem9uZSBvZiBjcm9wcywgbWluaW1pemluZyBsb3NzZXMgIGR1ZSB0byBldmFwb3JhdGlvbiwgcnVub2ZmLCBhbmQgZGVlcCBwZXJjb2xhdGlvbi4gQnkgIG1hdGNoaW5nIHdhdGVyIGFwcGxpY2F0aW9uIHJhdGVzIHRvIGNyb3Agd2F0ZXIgIHJlcXVpcmVtZW50cywgcHJlY2lzaW9uIGlycmlnYXRpb24gc3lzdGVtcyBlbnN1cmUgdGhhdCAgd2F0ZXIgaXMgdXNlZCBtb3JlIGVmZmljaWVudGx5LCBtYXhpbWl6aW5nIGNyb3AgeWllbGRzICBwZXIgdW5pdCBvZiB3YXRlciBhcHBsaWVkLiAyLiBFbmhhbmNlZCBDcm9wIFBlcmZvcm1hbmNlOiBCeSBwcm92aWRpbmcgY3JvcHMgd2l0aCBvcHRpbWFsIG1vaXN0dXJlIGxldmVscywgcHJlY2lzaW9uIGlycmlnYXRpb24gc3lzdGVtcyBwcm9tb3RlIGhlYWx0aHkgcm9vdCBkZXZlbG9wbWVudCwgYmFsYW5jZWQgZ3Jvd3RoLCAgYW5kIGltcHJvdmVkIGNyb3Agdmlnb3IuIENvbnNpc3RlbnQgc29pbCBtb2lzdHVyZSAgbGV2ZWxzIGhlbHAgcmVkdWNlIHdhdGVyIHN0cmVzcywgaW5jcmVhc2UgbnV0cmllbnQgIHVwdGFrZSwgYW5kIGVuaGFuY2UgY3JvcCByZXNpbGllbmNlIHRvIGVudmlyb25tZW50YWwgIHN0cmVzc2VzIHN1Y2ggYXMgZHJvdWdodCBhbmQgaGVhdC4gMy4gQ29zdCBTYXZpbmdzOiBQcmVjaXNpb24gaXJyaWdhdGlvbiBzeXN0ZW1zIGhlbHAgZmFybWVycyByZWR1Y2Ugd2F0ZXIsIGVuZXJneSwgYW5kIGxhYm9yIGNvc3RzIGFzc29jaWF0ZWQgd2l0aCBpcnJpZ2F0aW9uLiBCeSBvcHRpbWl6aW5nIHdhdGVyIHVzZSBlZmZpY2llbmN5IGFuZCAgbWluaW1pemluZyBpbnB1dCB3YXN0YWdlLCBwcmVjaXNpb24gaXJyaWdhdGlvbiBzeXN0ZW1zICBjYW4gbGVhZCB0byBzaWduaWZpY2FudCBjb3N0IHNhdmluZ3Mgb3ZlciB0aW1lLCAgaW1wcm92aW5nIHRoZSBwcm9maXRhYmlsaXR5IG9mIGFncmljdWx0dXJhbCBvcGVyYXRpb25zLiA0LiBFbnZpcm9ubWVudGFsIFN1c3RhaW5hYmlsaXR5OiAgUHJlY2lzaW9uICBpcnJpZ2F0aW9uIGNvbnRyaWJ1dGVzIHRvICBlbnZpcm9ubWVudGFsICBzdXN0YWluYWJpbGl0eSAgYnkgY29uc2VydmluZyB3YXRlciAgcmVzb3VyY2VzLCAgbWluaW1pemluZyAgd2F0ZXIgcG9sbHV0aW9uLCBhbmQgcmVkdWNpbmcgZ3JlZW5ob3VzZSBnYXMgZW1pc3Npb25zLiBCeSBhZG9wdGluZyBwcmVjaXNpb24gaXJyaWdhdGlvbiBwcmFjdGljZXMsIGZhcm1lcnMgY2FuICBtaW5pbWl6ZSB0aGVpciBlbnZpcm9ubWVudGFsIGZvb3RwcmludCBhbmQgY29udHJpYnV0ZSB0byB0aGUgIGxvbmctdGVybSAgc3VzdGFpbmFiaWxpdHkgIG9mICBhZ3JpY3VsdHVyYWwgcHJvZHVjdGlvbiBzeXN0ZW1zLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjQ0Ljg2NTY0MjU0NzYwNzQyLCJsZWZ0Ijo1My40MDgyNTI3MTYwNjQ0NSwid2lkdGgiOjM4LjI2MTc2ODM0MTA2NDQ1LCJoZWlnaHQiOjEuNjY5MjI3MzYxNjc5MDc3MX19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjQ2Ljg5NzkyMjUxNTg2OTE0LCJsZWZ0Ijo1Mi40MjA1NDM2NzA2NTQzLCJ3aWR0aCI6NDAuMjQwNzM3OTE1MDM5MDYsImhlaWdodCI6MS42MjU0NTQ3ODM0Mzk2MzYyfX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NDguNDkxNzkwNzcxNDg0Mzc1LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3Ljk4MDI3ODAxNTEzNjcyLCJoZWlnaHQiOjExLjAxMzcxMzgzNjY2OTkyMn19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjU5LjkzOTU0MDg2MzAzNzExLCJsZWZ0Ijo1Mi40MjA1NDM2NzA2NTQzLCJ3aWR0aCI6NDAuMjMwMjI0NjA5Mzc1LCJoZWlnaHQiOjEuNjI1NDU0NzgzNDM5NjM2Mn19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjYxLjUzMzQxNjc0ODA0Njg3NSwibGVmdCI6NTQuNjk4MzEwODUyMDUwNzgsIndpZHRoIjozNy45NjMxMjMzMjE1MzMyLCJoZWlnaHQiOjkuNDM1MTcwMTczNjQ1MDJ9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo3MS40MTY4Nzc3NDY1ODIwMywibGVmdCI6NTIuNDIwNTQzNjcwNjU0Mywid2lkdGgiOjQwLjIxMzMxMDI0MTY5OTIyLCJoZWlnaHQiOjEuNjI1NDU4MzU5NzE4MzIyOH19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjczLjAxMDc1NzQ0NjI4OTA2LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3Ljk0NTc4NTUyMjQ2MDk0LCJoZWlnaHQiOjcuODYzNzY0Mjg2MDQxMjZ9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo4MS4zMDg1NTU2MDMwMjczNCwibGVmdCI6NTIuNDIwNTQzNjcwNjU0Mywid2lkdGgiOjQwLjI0MDM3NTUxODc5ODgzLCJoZWlnaHQiOjEuNjI1NDU4MzU5NzE4MzIyOH19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjgyLjkxOTA1OTc1MzQxNzk3LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3LjkzNjQxNjYyNTk3NjU2LCJoZWlnaHQiOjEuNTc4MTMzNTgzMDY4ODQ3N319LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjg0LjQ4NjkwMDMyOTU4OTg0LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3LjkzNDc1NzIzMjY2NjAxNiwiaGVpZ2h0IjoxLjU3ODEzMzU4MzA2ODg0Nzd9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo4Ni4wNTQ3NDg1MzUxNTYyNSwibGVmdCI6NTQuNjk4MzEwODUyMDUwNzgsIndpZHRoIjozNy45Nzg0MzE3MDE2NjAxNTYsImhlaWdodCI6NC43MTM4MTkwMjY5NDcwMjE1fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6OC40MjAyOTI4NTQzMDkwODIsImxlZnQiOjEwLjAyMjE3MjkyNzg1NjQ0NSwid2lkdGgiOjM3Ljk0NjgzMDc0OTUxMTcyLCJoZWlnaHQiOjEuNTc4MTM1MjUxOTk4OTAxNH19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjkuOTg4MTMyNDc2ODA2NjQsImxlZnQiOjEwLjAyMjE3MjkyNzg1NjQ0NSwid2lkdGgiOjEzLjg2MjQ2MTA5MDA4Nzg5LCJoZWlnaHQiOjEuNTc4MTM1MjUxOTk4OTAxNH19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjI0OSJ9fV0%3D. However, the energy inputs for advanced irrigation systems, particularly for pressurized drip or sprinkler systems and desalination, can be significant, necessitating a comprehensive life cycle assessment of their environmental footprint.

### Soil Health and Salinity Management

Targeted watering prevents salinization from over-application, enhances microbial activity, and maintains structure, countering degradation in intensive systems (Timilsina et al., 2025)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjMwLCJzdWJ0eXBlIjp7fSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUmFqYSBSYWplbmRyYSIsImZhbWlseSI6IlRpbWlsc2luYSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTdW5pbCBLdW1hciIsImZhbWlseSI6Iktob3NsYSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJEaWwgQmFoYWR1ciIsImZhbWlseSI6IlJhaHV0In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlByYWR5b3QgUmFuamFuIiwiZmFtaWx5IjoiSmVuYSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJUZXRzdXNoaSIsImZhbWlseSI6IlNvbm9iZSJ9fV0sImlkcyI6eyJpc3NuIjpbIjIyOTYtNjY1WCJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMzM4OS9mZW52cy4yMDI1LjE1NDgwOTUiLCJvcGVuYWxleCI6Ilc0NDE0NzY1NTU4Iiwib3BlbkFjY2Vzc0lkIjoiMTU1MDU2MjIzMjI1NzI5MzI5MjAifSwidGl0bGUiOiJBIGNvbXByZWhlbnNpdmUgcmV2aWV3IG9mIHRoZSBzb2lsIGhlYWx0aCBzdGF0dXMgZm9yIGVuaGFuY2luZyBhZ3JpY3VsdHVyYWwgc3VzdGFpbmFiaWxpdHkiLCJhYnN0cmFjdCI6IlN1c3RhaW5hYmxlIGFncmljdWx0dXJhbCBwcmFjdGljZXMgaGF2ZSBiZWNvbWUgbW9yZSBjcnVjaWFsIHRoYW4gZXZlciBhcyB0aGUgd29ybGQgZ3JhcHBsZXMgd2l0aCBmb29kIGluc2VjdXJpdHkgYW5kIGVudmlyb25tZW50YWwgZGVncmFkYXRpb24uIFNvaWwgaGVhbHRoLCBhIGZ1bmRhbWVudGFsIGF0dHJpYnV0ZSBvZiBhZ3JpY3VsdHVyYWwgcHJvZHVjdGl2aXR5IGFuZCBlY29zeXN0ZW0gc3RhYmlsaXR5LCBwbGF5cyBhIHBpdm90YWwgcm9sZSBpbiBhY2hpZXZpbmcgZ2xvYmFsIHN1c3RhaW5hYmlsaXR5IHRhcmdldHMuIERlc3BpdGUgaXRzIGltcG9ydGFuY2UsIGNvbXByZWhlbnNpdmUgYW5hbHlzZXMgb2Ygc29pbCBoZWFsdGggaW5pdGlhdGl2ZXMgcmVtYWluIGxpbWl0ZWQuIFRoaXMgc3lzdGVtYXRpYyByZXZpZXcgYWRkcmVzc2VzIHRoaXMgZ2FwIHRocm91Z2ggYSBkdWFsIG9iamVjdGl2ZTogZmlyc3QsIHRvIGNvbnRleHR1YWxpemUgdGhlIGNyaXRpY2FsIHJvbGUgb2Ygc29pbCBoZWFsdGggaW4gYWR2YW5jaW5nIHRoZSBTdXN0YWluYWJsZSBEZXZlbG9wbWVudCBHb2FscyAoU0RHcyk7IGFuZCBzZWNvbmQsIHRvIGNyaXRpY2FsbHkgZXZhbHVhdGUgdGhlIGltcGxlbWVudGF0aW9uLCBpbXBhY3QsIGFuZCBwb2xpY3kgcmVsZXZhbmNlIG9mIEluZGlh4oCZcyBTb2lsIEhlYWx0aCBDYXJkIChTSEMpIHNjaGVtZSBsYXVuY2hlZCBpbiAyMDE1IHRvIGVuaGFuY2Ugc29pbCBmZXJ0aWxpdHkgdGhyb3VnaCBzaXRlLXNwZWNpZmljIGFzc2Vzc21lbnRzIGFuZCB0YWlsb3JlZCBmZXJ0aWxpemVyIHJlY29tbWVuZGF0aW9ucy4gR2xvYmFsIHJlc2VhcmNoIGhpZ2hsaWdodHMgdGhlIGNydWNpYWwgcm9sZSBvZiBzb2lsIGhlYWx0aCBpbiBwcm9tb3Rpbmcgc3VzdGFpbmFibGUgYWdyaWN1bHR1cmUsIGFuZCB0aGUgU0hDIHNjaGVtZSBoYXMgeWllbGRlZCBub3RhYmxlIHJlc3VsdHMsIGluY2x1ZGluZyBhIDUl4oCTNiUgaW5jcmVhc2UgaW4gY3JvcCB5aWVsZHMgYW5kIGFuIDgl4oCTMTAlIGRlY3JlYXNlIGluIGNoZW1pY2FsIGZlcnRpbGl6ZXIgdXNhZ2UuIEdyb3VuZGVkIGluIHRoZSBmcmFtZXdvcmsgb2YgdGhlIFNER3MsIHRoaXMgcGFwZXIgaGlnaGxpZ2h0cyB0aGUgU0hDIHNjaGVtZeKAmXMgY29udHJpYnV0aW9ucyB0byBlbmhhbmNpbmcgZmFybSBwcm9kdWN0aXZpdHksIGxvd2VyaW5nIGlucHV0IGNvc3RzLCBhbmQgYWR2YW5jaW5nIGVudmlyb25tZW50YWwgb2JqZWN0aXZlcy4gSXQgZnVydGhlciBleHBsb3JlcyB0aGUgcG9saWN5IGxhbmRzY2FwZSwgaWRlbnRpZmllcyBpbnN0aXR1dGlvbmFsIGdhcHMsIGFuZCBleGFtaW5lcyB0aGUgcG90ZW50aWFsIGZvciBzY2FsaW5nIGFuZCBhZGFwdGluZyB0aGVzZSBwcmFjdGljZXMgaW4gb3RoZXIgcmVnaW9ucy4gVGhpcyByZXZpZXcgb2ZmZXJzIGV2aWRlbmNlLWJhc2VkIGluc2lnaHRzIGludG8gdGhlIGludGVycGxheSBiZXR3ZWVuIHNvaWwgaGVhbHRoLCBhZ3JpY3VsdHVyYWwgc3VzdGFpbmFiaWxpdHksIGFuZCBwb2xpY3kgaW5ub3ZhdGlvbiwgYWltaW5nIHRvIGluZm9ybSBmdXR1cmUgaW50ZXJ2ZW50aW9ucyBhbmQgcHJvbW90ZSBpbnRlcm5hdGlvbmFsIGNvbGxhYm9yYXRpb25zIGluIHN1c3RhaW5hYmxlIGxhbmQgbWFuYWdlbWVudC4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMzM4OS9mZW52cy4yMDI1LjE1NDgwOTUiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vd3d3LmZyb250aWVyc2luLm9yZy9qb3VybmFscy9lbnZpcm9ubWVudGFsLXNjaWVuY2UvYXJ0aWNsZXMvMTAuMzM4OS9mZW52cy4yMDI1LjE1NDgwOTUvcGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjUtMTAtMDIiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTEwIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJGcm9udGllcnMgaW4gRW52aXJvbm1lbnRhbCBTY2llbmNlIiwidm9sdW1lIjoxMywiaW1wYWN0RmFjdG9yIjoiNS42NCJ9LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJGcm9udGllcnMgTWVkaWEifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjc5LCJjaXRlZEJ5Q291bnQiOjB9fSwicXVvdGUiOiIgcmVnaW9uYWwgZmFjdG9ycywgaW5jbHVkZSBiaW9sb2dpY2FsIGluZGljYXRvcnMsIGFuZCBhc3Nlc3MgbG9uZy10ZXJtIGltcGFjdHMgYW5kIHBvbGljeSBpbmNlbnRpdmVzIGZvciBhZG9wdGlvbiBQcmVjaXNpb24gZmFybWluZyBhbmQgbWFuYWdlbWVudCBWYW4gRGVuIEJlcmcgKDIwMDIpLCBWZXJtYSBhbmQgU2hhcm1hICgyMDA3KSwgVHVyaW5hd2UgZXQgYWwuICgyMDE1KSwgV3UgZXQgYWwuICgyMDE5KSwgWmhhbyBhbmQgV3UgKDIwMjEpLCBXYWRlIGV0IGFsLiAoMjAyMCksIFdpbGhlbG0gZXQgYWwuICgyMDIyKSwgWmhvdSBldCBhbC4gKDIwMjIpIFByZWNpc2lvbiBmYXJtaW5nIGJvb3N0cyBzb2lsIGhlYWx0aCBhbmQgeWllbGRzIHRocm91Z2ggdHJhaW5pbmcsIGluY2VudGl2ZXMsIGFuZCBkYXRhIHRvb2xzLCB5ZXQgcGVyc2lzdGVudCBpc3N1ZXMgbGlrZSBzYWxpbml0eSBhbmQgaGlnaCBjb3N0cyBleHBvc2UgaXRzIGxpbWl0ZWQgc2NhbGFiaWxpdHkgVGhlIGFic2VuY2Ugb2YgbG9uZy10ZXJtIGRhdGEsIGJpb2xvZ2ljYWwgaW5kaWNhdG9ycywgYW5kIHNvY2lvLWVjb25vbWljIGFuYWx5c2lzIHVuZGVybWluZXMgcmVzZWFyY2ggcmVsZXZhbmNlLCBzY2FsYWJpbGl0eSwgYW5kIHJlYWwtd29ybGQgYXBwbGljYWJpbGl0eSBGdXR1cmUgcmVzZWFyY2ggc2hvdWxkIGluY2x1ZGUgYmlvbG9naWNhbCBpbmRpY2F0b3JzLCBsb25nLXRlcm0gYW5kIHJlZ2lvbmFsIHN0dWRpZXMsIGFuZCBhZGRyZXNzIHNvY2lvLWVjb25vbWljIGFuZCBhZG9wdGlvbiBiYXJyaWVycyBmb3Igc2NhbGFibGUgcHJlY2lzaW9uIGZhcm1pbmcgU291cmNlOiBMaXRlcmF0dXJlIHN1cnZleS4gaW4gS2FybmF0YWthIChKb3NoaSBldCBhbC4sIDIwMTkpIGRlbW9uc3RyYXRlIHRoZSBlZmZlY3RpdmVuZXNzIG9mIHRhcmdldGVkIG51dHJpZW50IG1hbmFnZW1lbnQgdG9vbHMgbGlrZSBTSENzIGluIGltcHJvdmluZyB5aWVsZHMuIFRhaWxvcmVkIHpvbmFsIHN0cmF0ZWdpZXMgKE1laHJhIGFuZCBTaW5naCwgMjAxOCkgYW5kIHJpY2Ugc3lzdGVtLWZvY3VzZWQgcmVzZWFyY2ggKEJpc3dhcyBldCBhbC4sIDIwMTcpIGZ1cnRoZXIgdW5kZXJzY29yZSB0aGUgdmFsdWUgb2YgY29udGV4dC1zZW5zaXRpdmUgYXBwcm9hY2hlcy4gUmVjZW50IGxpdGVyYXR1cmUgaGFzIGFsc28gYnJvYWRlbmVkIHRvIGluY2x1ZGUgaW50ZXJkaXNjaXBsaW5hcnkgZXZhbHVhdGlvbnMgb2Ygc3VzdGFpbmFibGUgbGFuZCBtYW5hZ2VtZW50LiBSZXZpZXdzIGJ5IELDvG5lbWFubiBldCBhbC4gKDIwMTgpIGFuZCBCYWkgZXQgYWwuICgyMDE4KSBhZHZvY2F0ZSBmb3IgdXNpbmcgbXVsdGktaW5kaWNhdG9yIGZyYW1ld29ya3MgYW5kIHNpdGUtc3BlY2lmaWMgYmFzZWxpbmVzLiAiLCJzY29yZSI6MC4xOTc0MDAwMDM2NzE2NDYxMiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo2Mi4zMTY1NTg4Mzc4OTA2MjUsImxlZnQiOjcxLjk5OTkwMDgxNzg3MTEsIndpZHRoIjoxNy43ODY4NzI4NjM3Njk1MywiaGVpZ2h0IjowLjkxNDUwMTQ4ODIwODc3MDh9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo2My4zODA1ODA5MDIwOTk2MSwibGVmdCI6NzEuOTk5OTAwODE3ODcxMSwid2lkdGgiOjE4Ljg2NTYzNjgyNTU2MTUyMywiaGVpZ2h0IjowLjkxNDUwMTQ4ODIwODc3MDh9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo2NC40NDQ1OTUzMzY5MTQwNiwibGVmdCI6NzEuOTk5OTAwODE3ODcxMSwid2lkdGgiOjE4Ljk1Mzg0NTk3Nzc4MzIwMywiaGVpZ2h0IjowLjkxNDUwMTQ4ODIwODc3MDh9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo2NS41MTUzMTk4MjQyMTg3NSwibGVmdCI6NzEuOTk5OTAwODE3ODcxMSwid2lkdGgiOjUuOTM3NzUxMjkzMTgyMzczLCJoZWlnaHQiOjAuOTE0NTAxNDg4MjA4NzcwOH19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjY3LjUyMTgyMDA2ODM1OTM4LCJsZWZ0IjozOS45NzEzODk3NzA1MDc4MSwid2lkdGgiOjIwLjMwNjY4MDY3OTMyMTI5LCJoZWlnaHQiOjEuMDY1MDQzMDkxNzczOTg2OH19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjY5LjYyMjU3Mzg1MjUzOTA2LCJsZWZ0Ijo5LjA0NzU5OTc5MjQ4MDQ2OSwid2lkdGgiOjE5LjA0NjA1MTAyNTM5MDYyNSwiaGVpZ2h0Ijo1LjE3NzExMDE5NTE1OTkxMn19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjY5LjYyMjU3Mzg1MjUzOTA2LCJsZWZ0IjoyOS42NDc3OTA5MDg4MTM0NzcsIndpZHRoIjoxOS4xMDYwMjc2MDMxNDk0MTQsImhlaWdodCI6NS4xNzcxMTAxOTUxNTk5MTJ9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo2OS42MjI1NzM4NTI1MzkwNiwibGVmdCI6NTAuMzA0OTIwMTk2NTMzMiwid2lkdGgiOjIwLjE0Mjc5MzY1NTM5NTUwOCwiaGVpZ2h0Ijo0LjExMzA5NjIzNzE4MjYxN319LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjY5LjYyMjQ4OTkyOTE5OTIyLCJsZWZ0Ijo3Mi4wMDAxMzczMjkxMDE1Niwid2lkdGgiOjE4Ljk1NTY5MDM4MzkxMTEzMywiaGVpZ2h0Ijo1LjE3NzExMDE5NTE1OTkxMn19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjc1Ljc0MDQzMjczOTI1NzgxLCJsZWZ0Ijo4LjMzMzMyNzI5MzM5NTk5Niwid2lkdGgiOjExLjE3NjMzOTE0OTQ3NTA5OCwiaGVpZ2h0IjowLjg0OTE4MDkzNjgxMzM1NDV9fSx7InBhZ2VOdW1iZXIiOjYsImJib3giOnsidG9wIjo5LjU3NTM2OTgzNDg5OTkwMiwibGVmdCI6OC4zMzMzMjcyOTMzOTU5OTYsIndpZHRoIjo0MC40NzY2NTQwNTI3MzQzNzUsImhlaWdodCI6MS4xNzU3ODI1NjEzMDIxODV9fSx7InBhZ2VOdW1iZXIiOjYsImJib3giOnsidG9wIjoxMC45OTYyMTAwOTgyNjY2MDIsImxlZnQiOjguMzMzMzI5MjAwNzQ0NjI5LCJ3aWR0aCI6NDAuNDY3MTc0NTMwMDI5MywiaGVpZ2h0IjoxLjE3NTc4MjU2MTMwMjE4NX19LHsicGFnZU51bWJlciI6NiwiYmJveCI6eyJ0b3AiOjEyLjQxNzA1MDM2MTYzMzMsImxlZnQiOjguMzMzMzI5MjAwNzQ0NjI5LCJ3aWR0aCI6NDAuNDcxNjc5Njg3NSwiaGVpZ2h0IjoxLjE3NTc4MjU2MTMwMjE4NX19LHsicGFnZU51bWJlciI6NiwiYmJveCI6eyJ0b3AiOjEzLjgzNzg5MTU3ODY3NDMxNiwibGVmdCI6OC4zMzMzMjkyMDA3NDQ2MjksIndpZHRoIjo0MC40NzY0OTc2NTAxNDY0ODQsImhlaWdodCI6MS4xNzU3ODI1NjEzMDIxODV9fSx7InBhZ2VOdW1iZXIiOjYsImJib3giOnsidG9wIjoxNS4yNTg3MzA4ODgzNjY3LCJsZWZ0Ijo4LjMzMzMyNzI5MzM5NTk5Niwid2lkdGgiOjE3LjY4NDk0NjA2MDE4MDY2NCwiaGVpZ2h0IjoxLjE3NTc4MzUxNDk3NjUwMTV9fSx7InBhZ2VOdW1iZXIiOjYsImJib3giOnsidG9wIjoxNi42Nzk1NzExNTE3MzM0LCJsZWZ0IjoxMC42NzYxMzY5NzA1MjAwMiwid2lkdGgiOjM4LjEyNTExNDQ0MDkxNzk3LCJoZWlnaHQiOjEuMTc1NzgzNTE0OTc2NTAxNX19LHsicGFnZU51bWJlciI6NiwiYmJveCI6eyJ0b3AiOjE4LjA5MzcwNDIyMzYzMjgxMiwibGVmdCI6OC4zMzMzMjcyOTMzOTU5OTYsIndpZHRoIjo0MC40NzEwODQ1OTQ3MjY1NiwiaGVpZ2h0IjoxLjE3NTc4MzUxNDk3NjUwMTV9fSx7InBhZ2VOdW1iZXIiOjYsImJib3giOnsidG9wIjoxOS41MTQ1NDU0NDA2NzM4MjgsImxlZnQiOjguMzMzMzI5MjAwNzQ0NjI5LCJ3aWR0aCI6NDAuNDcwMzI1NDY5OTcwNywiaGVpZ2h0IjoxLjE3NTc4MzUxNDk3NjUwMTV9fSx7InBhZ2VOdW1iZXIiOjYsImJib3giOnsidG9wIjoyMC45MzUzODY2NTc3MTQ4NDQsImxlZnQiOjguMzMzMTc3NTY2NTI4MzIsIndpZHRoIjo0MC40NzA5MzIwMDY4MzU5NCwiaGVpZ2h0IjoxLjE3NTc4MzUxNDk3NjUwMTV9fV19LCJjaHVua0lkIjoiNTc1YTQ1NTAtMDA1Yi00YWFiLTg0NzUtZDdmYjBkNDhkNDQ3IiwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNiJ9fV0%3D. Proper drainage and integrated soil management practices are therefore essential to maximize the benefits of these advanced irrigation technologies while safeguarding soil productivity and preventing long-term ecological damage (Fader et al., 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1hcmlhbmVsYSIsImZhbWlseSI6IkZhZGVyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkNhcmxvIiwiZmFtaWx5IjoiR2l1cHBvbmkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2VsbWluIiwiZmFtaWx5IjoiQnVyYWsifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSGFtb3VkYSIsImZhbWlseSI6IkRha2hsYW91aSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBcmlzdGVpZGlzIiwiZmFtaWx5IjoiS291dHJvdWxpcyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNYW5mcmVkIEEuIiwiZmFtaWx5IjoiTGFuZ2UifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTWFyw61hIENhcm1lbiIsImZhbWlseSI6IkxsYXNhdCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJEYXZpZCIsImZhbWlseSI6IlB1bGlkb%2BKAkFZlbGF6cXVleiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBbGJlcnRvIiwiZmFtaWx5IjoiU2Fuei1Db2Jlw7FhIn19XSwiaWRzIjp7Imlzc24iOltdLCJpc2JuIjpbXSwib3BlbmFsZXgiOiJXNDI4NzU5NTQ0OCIsIm9wZW5BY2Nlc3NJZCI6IjExMjAxMTE3MTg2MjMyMjk0Mzk1In0sInRpdGxlIjoiQ2hhcHRlciAzIFJlc291cmNlcyAtIFN1YmNoYXB0ZXIgMy4xIFdhdGVyIiwidXJsIjoiaHR0cHM6Ly9oYWwtYW11LmFyY2hpdmVzLW91dmVydGVzLmZyL2hhbC0wMzIxMDA0MCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9hbXUuaGFsLnNjaWVuY2UvaGFsLTAzMjEwMDQwL2RvY3VtZW50IiwiaXNzdWVkRGF0ZSI6IjIwMjAtMTEtMTciLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA3IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJIQUwgKExlIENlbnRyZSBwb3VyIGxhIENvbW11bmljYXRpb24gU2NpZW50aWZpcXVlIERpcmVjdGUpIiwiaW1wYWN0RmFjdG9yIjoiMC4wNiJ9LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJDZW50cmUgTmF0aW9uYWwgZGUgbGEgUmVjaGVyY2hlIFNjaWVudGlmaXF1ZSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MCwiY2l0ZWRCeUNvdW50IjowfX0sInF1b3RlIjoiT3RoZXIgd2F0ZXIgY29uc2VydmF0aW9uICBzb2x1dGlvbnMsIHN0aWxsIHVuZGVyIGludmVzdGlnYXRpb24sIGluY2x1ZGUgcGxhbnQgIGNvbmRpdGlvbmVycyAoQm9hcmkgZXQgYWwuIDIwMTU7IMSGb3NpxIcgZXQgYWwuIDIwMTU7ICBDYW50b3JlIGV0IGFsLiAyMDE2OyBBYmRBbGxhaCBldCBhbC4gMjAxOCkgbGlrZSBhbi10aS10cmFuc3BpcmFudHMsIGJpby1zdGltdWxhbnRzIGFuZCBwbGFudCBncm93dGggIHJlZ3VsYXRvcnMsIHdoaWNoIHJlZ3VsYXRlIGNyb3AgdHJhbnNwaXJhdGlvbiBhbmQgIG1pdGlnYXRlIHRoZSBlZmZlY3RzIG9mIGFiaW90aWMgc3RyZXNzZXMsIGFuZCBzb2lsICBjb25kaXRpb25lcnMgKEd1aWxoZXJtZSBldCBhbC4gMjAxNSksIHdoaWNoIGFpbSB0byAgaW1wcm92ZSBzb2lsIHBoeXNpY2FsIHByb3BlcnRpZXMuIFNvbWUgdGVjaG5pcXVlcyBvZiBzdXN0YWluYWJsZSBpbnRlbnNpZmljYXRpb24sIHN1Y2ggYXMgbXVsY2hpbmcsIHplcm8gdGlsbGFnZSwgZXRjLiBpbmNyZWFzZSB0aGUgIHdhdGVyIHJldGVudGlvbiBjYXBhY2l0eSBvZiBzb2lscyBtYWtpbmcgdGhlbSAgbW9yZSBjYXBhYmxlIG9mIGNvcGluZyB3aXRoIGRyeSBzcGVsbHMgYW5kIGluLWNyZWFzaW5nIHRoZSB3YXRlciBhbW91bnQgYWNjZXNzaWJsZSB0byBwbGFudHMgIChLYXNzYW0gZXQgYWwuIDIwMTIpIChTZWN0aW9uIDMuMi4zLjEgYW5kIDYuNCkuICBBbHNvLCBtb3JlIGVmZmljaWVudCBpcnJpZ2F0aW9uIHN5c3RlbXMsIHNoaWZ0cyB0by13YXJkcyBkcm91Z2h0IHRvbGVyYW50IGNyb3BzLCBhZGFwdGF0aW9uIG9mIHNvdy1pbmcgZGF0ZXMsIGFwcGxpY2F0aW9uIG9mIGRlZmljaXQgaXJyaWdhdGlvbiBzY2hlbWVzLCAgbGFuZCByZWNsYW1hdGlvbiwgYW5kIGxhbmQgbWFuYWdlbWVudCBmb3IgY2FyLWJvbiBzZXF1ZXN0cmF0aW9uIChBbG1hZ3JvIGV0IGFsLiAyMDE2OyBGdW5lcyBldCAxMiBodHRwOi8vbWVkYWNjLWxpZmUuZXUvIGFsLiAyMDE5KSBtYXkgcmVkdWNlIHdhdGVyIG5lZWRzIGZvciBhZ3JpY3VsdHVyZSBhbmQgaW5jcmVhc2Ugd2F0ZXIgdXNlIGVmZmljaWVuY3kgaW4gdGVybXMgb2YgbTMgIHBlciB0b25zLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MzYsImJib3giOnsidG9wIjo1OC43MDcyMTgxNzAxNjYwMTYsImxlZnQiOjQ5Ljg4ODg2MjYwOTg2MzI4LCJ3aWR0aCI6MzcuMzY0ODcxOTc4NzU5NzY2LCJoZWlnaHQiOjEuNDM4NDI4NDAxOTQ3MDIxNX19LHsicGFnZU51bWJlciI6MzYsImJib3giOnsidG9wIjo2MC4xMzI1ODc0MzI4NjEzMywibGVmdCI6NDkuODg4ODYyNjA5ODYzMjgsIndpZHRoIjozNy4zNjU1MTY2NjI1OTc2NTYsImhlaWdodCI6MS40Mzg0Mjg0MDE5NDcwMjE1fX0seyJwYWdlTnVtYmVyIjozNiwiYmJveCI6eyJ0b3AiOjYxLjU1Nzk0OTA2NjE2MjExLCJsZWZ0Ijo0OS44ODg4NjI2MDk4NjMyOCwid2lkdGgiOjM3LjM2NDg3MTk3ODc1OTc2NiwiaGVpZ2h0IjoxLjQzODQyODQwMTk0NzAyMTV9fSx7InBhZ2VOdW1iZXIiOjM2LCJiYm94Ijp7InRvcCI6NjIuOTgzMzE0NTE0MTYwMTU2LCJsZWZ0Ijo0OS44ODg4NjI2MDk4NjMyOCwid2lkdGgiOjM2LjkzNjQ5NjczNDYxOTE0LCJoZWlnaHQiOjEuNDM4NDI4NDAxOTQ3MDIxNX19LHsicGFnZU51bWJlciI6MzYsImJib3giOnsidG9wIjo2NC40MDg2NzYxNDc0NjA5NCwibGVmdCI6NDkuODg4ODYyNjA5ODYzMjgsIndpZHRoIjozNy4zNjIxNTIwOTk2MDkzNzUsImhlaWdodCI6MS40Mzg0Mjg0MDE5NDcwMjE1fX0seyJwYWdlTnVtYmVyIjozNiwiYmJveCI6eyJ0b3AiOjY1LjgzNDA0NTQxMDE1NjI1LCJsZWZ0Ijo0OS44ODg4NjI2MDk4NjMyOCwid2lkdGgiOjM3LjM2MzQ5NDg3MzA0Njg3NSwiaGVpZ2h0IjoxLjQzODQyODQwMTk0NzAyMTV9fSx7InBhZ2VOdW1iZXIiOjM2LCJiYm94Ijp7InRvcCI6NjcuMjU5NDA3MDQzNDU3MDMsImxlZnQiOjQ5Ljg4ODg2MjYwOTg2MzI4LCJ3aWR0aCI6MzcuMzY3MzcwNjA1NDY4NzUsImhlaWdodCI6MS40Mzg0Mjg0MDE5NDcwMjE1fX0seyJwYWdlTnVtYmVyIjozNiwiYmJveCI6eyJ0b3AiOjY4LjY4NDc3NjMwNjE1MjM0LCJsZWZ0Ijo0OS44ODg4NjI2MDk4NjMyOCwid2lkdGgiOjM3LjM2NDg0OTA5MDU3NjE3LCJoZWlnaHQiOjEuNDM4NDI4NDAxOTQ3MDIxNX19LHsicGFnZU51bWJlciI6MzYsImJib3giOnsidG9wIjo3MC4xMTAxMzc5Mzk0NTMxMiwibGVmdCI6NDkuODg4ODYyNjA5ODYzMjgsIndpZHRoIjoyMy43NDk5OTYxODUzMDI3MzQsImhlaWdodCI6MS40Mzg0Mjg0MDE5NDcwMjE1fX0seyJwYWdlTnVtYmVyIjozNiwiYmJveCI6eyJ0b3AiOjcyLjk2MDg2ODgzNTQ0OTIyLCJsZWZ0Ijo0OS44ODg4NjI2MDk4NjMyOCwid2lkdGgiOjM3LjM2NTY4MDY5NDU4MDA4LCJoZWlnaHQiOjE1LjY5MjA3MTkxNDY3Mjg1Mn19LHsicGFnZU51bWJlciI6MzYsImJib3giOnsidG9wIjo5MC40NDkxMDQzMDkwODIwMywibGVmdCI6MTAuMDk1MjMzOTE3MjM2MzI4LCJ3aWR0aCI6MTIuMzA5NDE0ODYzNTg2NDI2LCJoZWlnaHQiOjEuMDA2ODk5ODMzNjc5MTk5Mn19LHsicGFnZU51bWJlciI6MzcsImJib3giOnsidG9wIjoxMC4yNDQ4MzQ4OTk5MDIzNDQsImxlZnQiOjEzLjE3NDU5NDg3OTE1MDM5LCJ3aWR0aCI6MzcuMzY0ODQ1Mjc1ODc4OTA2LCJoZWlnaHQiOjEuNDM4NDMwMTkwMDg2MzY0N319LHsicGFnZU51bWJlciI6MzcsImJib3giOnsidG9wIjoxMS42NzAxOTkzOTQyMjYwNzQsImxlZnQiOjEzLjE3NDU5NDg3OTE1MDM5LCJ3aWR0aCI6MzcuMzY0ODQ5MDkwNTc2MTcsImhlaWdodCI6MS40Mzg0MzAxOTAwODYzNjQ3fX0seyJwYWdlTnVtYmVyIjozNywiYmJveCI6eyJ0b3AiOjEzLjA5NTU2Mzg4ODU0OTgwNSwibGVmdCI6MTMuMTc0NTk0ODc5MTUwMzksIndpZHRoIjozNy4zNjczNzA2MDU0Njg3NSwiaGVpZ2h0IjoxLjQzODQzMDE5MDA4NjM2NDd9fV19LCJjaHVua0lkIjoiOTNhM2VhNDktMmZjOC00NDU0LTkxZmItNzlkNTU0MjQzYzBkIiwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiMzcifX1d.

### Economic Viability and Farmer Adoption

Cost savings on inputs (water, energy, labor) improve profitability, though upfront investments necessitate subsidies; scalability favors smallholders via modular designs (Kushwaha et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjMwLCJzdWJ0eXBlIjp7fSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTWFuaXNoIiwiZmFtaWx5IjoiS3VzaHdhaGEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2hhbmthciIsImZhbWlseSI6IlNpbmdoIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlZpamF5IEthbnQiLCJmYW1pbHkiOiJTaW5naCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTaGFzaGFuayIsImZhbWlseSI6IkR3aXZlZGkifX1dLCJpZHMiOnsiaXNzbiI6WyIyNDU2LTE4NzgiXSwiaXNibiI6W10sImRvaSI6IjEwLjIyMTYxL2lqZWFiLjkyLjI3Iiwib3BlbmFsZXgiOiJXNDM5NjU2MzU4NCJ9LCJ0aXRsZSI6IlByZWNpc2lvbiBGYXJtaW5nOiBBIFJldmlldyBvZiBNZXRob2RzLCBUZWNobm9sb2dpZXMsIGFuZCBGdXR1cmUgUHJvc3BlY3RzIiwiYWJzdHJhY3QiOiJQcmVjaXNpb24gZmFybWluZywgb2Z0ZW4gcmVmZXJyZWQgdG8gYXMgcHJlY2lzaW9uIGFncmljdWx0dXJlLCBpcyBhIGZhcm1pbmcgbWFuYWdlbWVudCBjb25jZXB0IHRoYXQgc2Vla3MgdG8gbWF4aW1pemUgYWdyaWN1bHR1cmFsIG9wZXJhdGlvbnMgdmlhIHRoZSB1c2Ugb2YgdGVjaG5vbG9neSBpbiBvcmRlciB0byBhY2hpZXZlIGdyZWF0ZXIgbGV2ZWxzIG9mIGVmZmljaWVuY3ksIHByb2R1Y3Rpb24sIGFuZCBjb25zZXJ2YXRpb24gb2YgcmVzb3VyY2VzLiBUaGUgcHVycG9zZSBvZiB0aGlzIHN1cnZleSBzdHVkeSBpcyB0byBvZmZlciBhbiBvdmVydmlldyBvZiBwcmVjaXNpb24gZmFybWluZywgaW5jbHVkaW5nIHRvcGljcyBzdWNoIGFzIGl0cyBmdW5kYW1lbnRhbHMsIG1ldGhvZG9sb2dpZXMsIHRlY2hub2xvZ3ksIGFuZCBhcHBsaWNhdGlvbnMuIEl0IGludmVzdGlnYXRlcyB0aGUgcm9sZSB0aGF0IGEgdmFyaWV0eSBvZiB0ZWNobm9sb2dpZXMsIGluY2x1ZGluZyBnbG9iYWwgbmF2aWdhdGlvbiBzYXRlbGxpdGUgc3lzdGVtcyAoR05TUyksIGdlb2dyYXBoaWMgaW5mb3JtYXRpb24gc3lzdGVtcyAoR0lTKSwgcmVtb3RlIHNlbnNpbmcsIGFuZCBzZW5zb3IgdGVjaG5vbG9neSwgcGxheSBpbiBtYWtpbmcgaXQgcG9zc2libGUgdG8gaW1wbGVtZW50IHByZWNpc2lvbiBhZ3JpY3VsdHVyYWwgbWV0aG9kcy4gSW4gYWRkaXRpb24sIGl0IGFkZHJlc3NlcyB0aGUgYWR2YW50YWdlcywgZGlzYWR2YW50YWdlcywgYW5kIHBvdGVudGlhbCBmdXR1cmUgYXBwbGljYXRpb25zIG9mIHByZWNpc2lvbiBmYXJtaW5nIGluIHRoZSBjb250ZXh0IG9mIHNvbHZpbmcgaXNzdWVzIHJlbGF0ZWQgdG8gdGhlIHN1c3RhaW5hYmlsaXR5IG9mIGFncmljdWx0dXJlIGFuZCB0aGUgc2VjdXJpdHkgb2YgZm9vZCBzdXBwbGllcyBvbiBhIHdvcmxkd2lkZSBzY2FsZS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjIxNjEvaWplYWIuOTIuMjciLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vaWplYWIuY29tL3VwbG9hZF9kb2N1bWVudC9pc3N1ZV9maWxlcy8yN0lKRUFCLTEwNDIwMjQ4NC1QcmVjaXNpb25GYXJtaW5nLnBkZiIsImlzc3VlZERhdGUiOiIyMDI0LTAxLTAxIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiSW50ZXJuYXRpb25hbCBKb3VybmFsIG9mIEVudmlyb25tZW50IEFncmljdWx0dXJlIGFuZCBCaW90ZWNobm9sb2d5IiwiaXNzdWUiOjIsInZvbHVtZSI6OSwiaW1wYWN0RmFjdG9yIjoiMC42NSJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjI0MiwibGFzdFBhZ2UiOjI1M319LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJJbmZvZ2FpbiBQdWJsaWNhdGlvbiJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NCwiY2l0ZWRCeUNvdW50Ijo4fX0sInF1b3RlIjoiSUlJLiBCRU5FRklUUyBPRiBQUkVDSVNJT04gSVJSSUdBVElPTiAxLiBJbXByb3ZlZCBXYXRlciBVc2UgRWZmaWNpZW5jeTogUHJlY2lzaW9uIGlycmlnYXRpb24gb3B0aW1pemVzIHdhdGVyIHVzZSBlZmZpY2llbmN5IGJ5IGRlbGl2ZXJpbmcgd2F0ZXIgZGlyZWN0bHkgdG8gdGhlIHJvb3Qgem9uZSBvZiBjcm9wcywgbWluaW1pemluZyBsb3NzZXMgIGR1ZSB0byBldmFwb3JhdGlvbiwgcnVub2ZmLCBhbmQgZGVlcCBwZXJjb2xhdGlvbi4gQnkgIG1hdGNoaW5nIHdhdGVyIGFwcGxpY2F0aW9uIHJhdGVzIHRvIGNyb3Agd2F0ZXIgIHJlcXVpcmVtZW50cywgcHJlY2lzaW9uIGlycmlnYXRpb24gc3lzdGVtcyBlbnN1cmUgdGhhdCAgd2F0ZXIgaXMgdXNlZCBtb3JlIGVmZmljaWVudGx5LCBtYXhpbWl6aW5nIGNyb3AgeWllbGRzICBwZXIgdW5pdCBvZiB3YXRlciBhcHBsaWVkLiAyLiBFbmhhbmNlZCBDcm9wIFBlcmZvcm1hbmNlOiBCeSBwcm92aWRpbmcgY3JvcHMgd2l0aCBvcHRpbWFsIG1vaXN0dXJlIGxldmVscywgcHJlY2lzaW9uIGlycmlnYXRpb24gc3lzdGVtcyBwcm9tb3RlIGhlYWx0aHkgcm9vdCBkZXZlbG9wbWVudCwgYmFsYW5jZWQgZ3Jvd3RoLCAgYW5kIGltcHJvdmVkIGNyb3Agdmlnb3IuIENvbnNpc3RlbnQgc29pbCBtb2lzdHVyZSAgbGV2ZWxzIGhlbHAgcmVkdWNlIHdhdGVyIHN0cmVzcywgaW5jcmVhc2UgbnV0cmllbnQgIHVwdGFrZSwgYW5kIGVuaGFuY2UgY3JvcCByZXNpbGllbmNlIHRvIGVudmlyb25tZW50YWwgIHN0cmVzc2VzIHN1Y2ggYXMgZHJvdWdodCBhbmQgaGVhdC4gMy4gQ29zdCBTYXZpbmdzOiBQcmVjaXNpb24gaXJyaWdhdGlvbiBzeXN0ZW1zIGhlbHAgZmFybWVycyByZWR1Y2Ugd2F0ZXIsIGVuZXJneSwgYW5kIGxhYm9yIGNvc3RzIGFzc29jaWF0ZWQgd2l0aCBpcnJpZ2F0aW9uLiBCeSBvcHRpbWl6aW5nIHdhdGVyIHVzZSBlZmZpY2llbmN5IGFuZCAgbWluaW1pemluZyBpbnB1dCB3YXN0YWdlLCBwcmVjaXNpb24gaXJyaWdhdGlvbiBzeXN0ZW1zICBjYW4gbGVhZCB0byBzaWduaWZpY2FudCBjb3N0IHNhdmluZ3Mgb3ZlciB0aW1lLCAgaW1wcm92aW5nIHRoZSBwcm9maXRhYmlsaXR5IG9mIGFncmljdWx0dXJhbCBvcGVyYXRpb25zLiA0LiBFbnZpcm9ubWVudGFsIFN1c3RhaW5hYmlsaXR5OiAgUHJlY2lzaW9uICBpcnJpZ2F0aW9uIGNvbnRyaWJ1dGVzIHRvICBlbnZpcm9ubWVudGFsICBzdXN0YWluYWJpbGl0eSAgYnkgY29uc2VydmluZyB3YXRlciAgcmVzb3VyY2VzLCAgbWluaW1pemluZyAgd2F0ZXIgcG9sbHV0aW9uLCBhbmQgcmVkdWNpbmcgZ3JlZW5ob3VzZSBnYXMgZW1pc3Npb25zLiBCeSBhZG9wdGluZyBwcmVjaXNpb24gaXJyaWdhdGlvbiBwcmFjdGljZXMsIGZhcm1lcnMgY2FuICBtaW5pbWl6ZSB0aGVpciBlbnZpcm9ubWVudGFsIGZvb3RwcmludCBhbmQgY29udHJpYnV0ZSB0byB0aGUgIGxvbmctdGVybSAgc3VzdGFpbmFiaWxpdHkgIG9mICBhZ3JpY3VsdHVyYWwgcHJvZHVjdGlvbiBzeXN0ZW1zLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjQ0Ljg2NTY0MjU0NzYwNzQyLCJsZWZ0Ijo1My40MDgyNTI3MTYwNjQ0NSwid2lkdGgiOjM4LjI2MTc2ODM0MTA2NDQ1LCJoZWlnaHQiOjEuNjY5MjI3MzYxNjc5MDc3MX19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjQ2Ljg5NzkyMjUxNTg2OTE0LCJsZWZ0Ijo1Mi40MjA1NDM2NzA2NTQzLCJ3aWR0aCI6NDAuMjQwNzM3OTE1MDM5MDYsImhlaWdodCI6MS42MjU0NTQ3ODM0Mzk2MzYyfX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NDguNDkxNzkwNzcxNDg0Mzc1LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3Ljk4MDI3ODAxNTEzNjcyLCJoZWlnaHQiOjExLjAxMzcxMzgzNjY2OTkyMn19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjU5LjkzOTU0MDg2MzAzNzExLCJsZWZ0Ijo1Mi40MjA1NDM2NzA2NTQzLCJ3aWR0aCI6NDAuMjMwMjI0NjA5Mzc1LCJoZWlnaHQiOjEuNjI1NDU0NzgzNDM5NjM2Mn19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjYxLjUzMzQxNjc0ODA0Njg3NSwibGVmdCI6NTQuNjk4MzEwODUyMDUwNzgsIndpZHRoIjozNy45NjMxMjMzMjE1MzMyLCJoZWlnaHQiOjkuNDM1MTcwMTczNjQ1MDJ9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo3MS40MTY4Nzc3NDY1ODIwMywibGVmdCI6NTIuNDIwNTQzNjcwNjU0Mywid2lkdGgiOjQwLjIxMzMxMDI0MTY5OTIyLCJoZWlnaHQiOjEuNjI1NDU4MzU5NzE4MzIyOH19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjczLjAxMDc1NzQ0NjI4OTA2LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3Ljk0NTc4NTUyMjQ2MDk0LCJoZWlnaHQiOjcuODYzNzY0Mjg2MDQxMjZ9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo4MS4zMDg1NTU2MDMwMjczNCwibGVmdCI6NTIuNDIwNTQzNjcwNjU0Mywid2lkdGgiOjQwLjI0MDM3NTUxODc5ODgzLCJoZWlnaHQiOjEuNjI1NDU4MzU5NzE4MzIyOH19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjgyLjkxOTA1OTc1MzQxNzk3LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3LjkzNjQxNjYyNTk3NjU2LCJoZWlnaHQiOjEuNTc4MTMzNTgzMDY4ODQ3N319LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjg0LjQ4NjkwMDMyOTU4OTg0LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3LjkzNDc1NzIzMjY2NjAxNiwiaGVpZ2h0IjoxLjU3ODEzMzU4MzA2ODg0Nzd9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo4Ni4wNTQ3NDg1MzUxNTYyNSwibGVmdCI6NTQuNjk4MzEwODUyMDUwNzgsIndpZHRoIjozNy45Nzg0MzE3MDE2NjAxNTYsImhlaWdodCI6NC43MTM4MTkwMjY5NDcwMjE1fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6OC40MjAyOTI4NTQzMDkwODIsImxlZnQiOjEwLjAyMjE3MjkyNzg1NjQ0NSwid2lkdGgiOjM3Ljk0NjgzMDc0OTUxMTcyLCJoZWlnaHQiOjEuNTc4MTM1MjUxOTk4OTAxNH19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjkuOTg4MTMyNDc2ODA2NjQsImxlZnQiOjEwLjAyMjE3MjkyNzg1NjQ0NSwid2lkdGgiOjEzLjg2MjQ2MTA5MDA4Nzg5LCJoZWlnaHQiOjEuNTc4MTM1MjUxOTk4OTAxNH19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjI0OSJ9fV0%3D. Furthermore, the integration of genomic, agronomic, and environmental predictors within novel G x M technologies offers a framework for developing improved crop varieties that exhibit enhanced resource use efficiency, thereby fostering food security and nutrition (Sharma et al., 2025)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjMwLCJzdWJ0eXBlIjp7fSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUmFtYW5kZWVwIEt1bWFyIiwiZmFtaWx5IjoiU2hhcm1hIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5rZW0gSi4iLCJmYW1pbHkiOiJOd29zdSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJMb3ZlcHJlZXQiLCJmYW1pbHkiOiJTaW5naCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBbWFuZGEiLCJmYW1pbHkiOiJLcmFtZXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSGFyZGVlcCIsImZhbWlseSI6IlNpbmdoIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlJhanUiLCJmYW1pbHkiOiJCaGVlbWFuYWhhbGxpIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkphZ21hbmRlZXAiLCJmYW1pbHkiOiJEaGlsbG9uIn19XSwiaWRzIjp7Imlzc24iOlsiMjczMS05NTk4Il0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDA3L3M0NDI3OS0wMjUtMDAyODctNCIsIm9wZW5hbGV4IjoiVzQ0MTMyMjIzODIiLCJvcGVuQWNjZXNzSWQiOiIxMjMyNjI0NTc3NTUzNzYyNDUzNyJ9LCJ0aXRsZSI6IkFncmljdWx0dXJhbCBzdXN0YWluYWJpbGl0eSB1bmRlciB1bnByZWRpY3RlZCBhdG1vc3BoZXJpYyBjaGFuZ2Vz4oCUc3RyYXRlZ2llcyB0byBlbmhhbmNlIGNyb3AgcmVzaWxpZW5jZSBhbmQgc3lzdGVtIGVmZmljaWVuY3k6IGEgbmFycmF0aXZlIHJldmlldyIsImFic3RyYWN0IjoiRWFydGggaXMgd2FybWVyIGJ5IDAuODUgwrBDIGNvbXBhcmVkIHRvIHRoZSBwcmV2aW91cyBjZW50dXJ5LCBldmlkZW5jaW5nIHRoYXQgdGhlIGF0bW9zcGhlcmljIGNvbmRpdGlvbnMgYXJlIHJhcGlkbHkgY2hhbmdpbmcsIHRocmVhdGVuaW5nIGFncmljdWx0dXJlIGFuZCBnbG9iYWwgZm9vZCBzZWN1cml0eS4gVGhpcyBjbGltYXRpYyBjaGFuZ2UgaW5kdWNlcyBiaW90aWMgYW5kIGFiaW90aWMgc3RyZXNzLCBhbHRlcnMgcGxhbnRzJyBtaWNyb2NsaW1hdGVzLCBhbmQgaW5mbHVlbmNlcyBwbGFudCBwaHlzaW9sb2dpY2FsIGFuZCBtb3JwaG9sb2dpY2FsIHByb2Nlc3NlcywgY29uc2VxdWVudGx5IGltcGFjdGluZyBjcm9wIHByb2R1Y3Rpdml0eSBhbmQgZmFybSBwcm9maXRhYmlsaXR5LiBUaGUgcHJpbWFyeSByZWFzb24gZm9yIHRoZSBjbGltYXRpYyBzaGlmdHMgaXMgdGhlIGluY3JlYXNlZCBncmVlbmhvdXNlIGdhcyAoR0hHKSBlbWlzc2lvbnMsIG1haW5seSBjYXJib24sIGluIHRoZSBlbnZpcm9ubWVudCwgcmVzdWx0aW5nIGZyb20gY2VtZW50IG1hbnVmYWN0dXJpbmcsIGRlZm9yZXN0YXRpb24sIGFuZCBidXJuaW5nIG9mIGZvc3NpbCBmdWVscywgYW1vbmcgb3RoZXIgZmFjdG9ycy4gVGhlcmVmb3JlLCBsb3dlcmluZyBjYXJib24gZW1pc3Npb25zIG9jY3VwaWVzIGEgdml0YWwgcGxhY2UgaW4gZ2xvYmFsIGJ1ZGdldGluZywgd2l0aCBpdHMgc29jaWFsIGNvc3QgKCQyOSB0b27iiJIgMSBvZiBjYXJib24pIGluY3JlYXNpbmcgYXQgMiUgeWVhcuKIkiAxLiBUaGlzIHJldmlldyBzdW1tYXJpemVzIHRoZSBsaXRlcmF0dXJlIHJlbGF0aW5nIHRvIGNsaW1hdGUgY2hhbmdlLCBjb25zaWRlcmluZyBib3RoIGFudGhyb3BvZ2VuaWMgYW5kIG5hdHVyYWwgc291cmNlcywgYW5kIGl0cyBtdWx0aWZhY2V0ZWQgaW1wbGljYXRpb25zIGZvciBhZ3JpY3VsdHVyZS4gSXQgdGhlbiBleHBhbmRzIG9uIHRoZSBpbXBhY3Qgb2YgY2xpbWF0ZSBjaGFuZ2Ugb24gcGxhbnQgbW9ycGhvbG9neSwgcGxhbnQgcGh5c2lvbG9neSwgcGxhbnQgcHJvZHVjdGl2aXR5LCBzb2lsIGh5ZHJvbG9neSwgc29pbCBmZXJ0aWxpdHksIGlycmlnYXRpb24gYXZhaWxhYmlsaXR5LCBhbmQgc29jaW9lY29ub21pY3MuIFRoZW4gaXQgZXhwbG9yZWQgdGhlIGZ1dHVyZSByZXNlYXJjaCBkaXJlY3Rpb25zIHRhcmdldGVkIGF0IGltcHJvdmluZyB0aGUgY3VycmVudCBjbGltYXRlLWNyb3AgYXNzZXNzbWVudCBtZXRob2RzLiBGdXJ0aGVybW9yZSwgdGhpcyByZXZpZXcgaWxsdXN0cmF0ZXMgdGhlIHBvc3NpYmxlIG1pdGlnYXRpb24gc3RyYXRlZ2llcyBjb25zaWRlcmluZyB0aGUgd2F0ZXIsIG51dHJpZW50LCB3ZWF0aGVyLCBjYXJib24sIGFuZCBrbm93bGVkZ2Utc21hcnQgYWdyaWN1bHR1cmUgc3VjaCBhcyAoaSkgc3VzdGFpbmFibGUgbGFuZCBtYW5hZ2VtZW50LCAoaWkpIHNvaWwgd2F0ZXIgcmV0ZW50aW9uIGFuZCBzb2lsIGNvbnNlcnZhdGlvbiwgKGlpaSkgc29pbCBudXRyaWVudCByZXRlbnRpb24gcHJhY3RpY2VzLCAoaXYpIHJlZHVjdGlvbiBvZiBHSEdzLCAodikgY2xpbWF0ZSBzbWFydCBhZ3JpY3VsdHVyZSBhbmQgdXNlIG9mIGltcHJvdmVkIGFncmljdWx0dXJhbCB0ZWNobm9sb2dpZXMuIExhc3RseSwgdGhpcyByZXZpZXcgZWx1Y2lkYXRlcyB0aGUgdHJhbnNmb3JtYXRpdmUgcHJvZ3Jlc3MgaW4gcGxhbnQgYnJlZWRpbmcsIGdlbmV0aWNzLCBhbmQgZ2Vub21pYyBzdHJhdGVnaWVzIG9mIGNyb3AgaW1wcm92ZW1lbnQsIHN1Y2ggYXMgKGkpIGdlbm90eXBlIGJ5IGVudmlyb25tZW50IGludGVyYWN0aW9ucyBhbmQgcHJlZGljdGl2ZSBicmVlZGluZywgKGlpKSBhZGFwdGl2ZSB0cmFpdHMsIGhpZ2gtdGhyb3VnaHB1dCBwaGVub3R5cGluZyBhbmQgcGhlbm9taWNzLCBhbmQgKGlpaSkgbW9sZWN1bGFyIGFwcHJvYWNoZXMgZm9yIHN0dWR5aW5nIHN0cmVzcyB0b2xlcmFuY2UuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjEwMDcvczQ0Mjc5LTAyNS0wMDI4Ny00IiwicmVzb3VyY2VVcmwiOiJodHRwczovL2xpbmsuc3ByaW5nZXIuY29tL2NvbnRlbnQvcGRmLzEwLjEwMDcvczQ0Mjc5LTAyNS0wMDI4Ny00LnBkZiIsImlzc3VlZERhdGUiOiIyMDI1LTA4LTExIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiRGlzY292ZXIgQWdyaWN1bHR1cmUiLCJpc3N1ZSI6MSwidm9sdW1lIjozLCJpbXBhY3RGYWN0b3IiOiIyLjEzIn0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlNwcmluZ2VyIFNjaWVuY2UrQnVzaW5lc3MgTWVkaWEifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjI3MCwiY2l0ZWRCeUNvdW50IjowfX0sInF1b3RlIjoiSG93ZXZlciwgZ2Vuby10eXBlIChHKSBhbmQgZW52aXJvbm1lbnQgKEUpLCBhbmQgbWFuYWdlbWVudCAoTSkgKEcgeCBFIHggTSkgaW50ZXJhY3Rpb25zIGFyZSBwcmVzZW50ICBpbiBhZ3JpY3VsdHVyZS4gQWRkaXRpb25hbGx5LCBjaG9vc2luZyBnZW5vdHlwZXMgZm9yIGFtYmlndW91cyBUUEUgYW5kL29yIG1hbmFnZW1lbnQgIGludmFyaWFibHkgcmVzdWx0cyBpbiBsb3dlciByYXRlcyBvZiBnZW5ldGljIGdhaW4gdGhhbiBhcmUgcG9zc2libGUsIGFuZCB0aGVyZSBpcyBldmlkZW5jZSAgdG8gc3VnZ2VzdCB0aGF0IGNsaW1hdGUgY2hhbmdlIGhpbmRlcmVkIGdlbmV0aWMgZ2FpbiBpbiByb3cgY3JvcHMsIHNwZWNpZmljYWxseSBpbiB3aGVhdCAgWzI4N%2BKAkzI4OV0uIFRoZXJlIGlzIGEgbGFjayBvZiBhIGZyYW1ld29yayBmb3IgYnJlZWRpbmcgY3JvcHMsIGFsb25nIHdpdGggdGhlIG1lbnRpb25lZCAgcHJvYmxlbXMgb2YgRyB4IEUgeCBNIGludGVyYWN0aW9ucyBhbmQgdGhlIHRpbWUgbGFnIG9mIHZhcmlldHkgZGV2ZWxvcG1lbnQuIFJlY2VudGx5LCBhICBmcmFtZXdvcmsgd2FzIHByb3Bvc2VkIHRvIGRldmVsb3AgbmV3IEcgeCBNIHRlY2hub2xvZ2llcywgd2hpY2ggaW50ZWdyYXRlIGdlbm9taWMsICBhZ3Jvbm9taWMsIGFuZCBlbnZpcm9ubWVudGFsIHByZWRpY3RvcnMgdG8gcHJlZGljdCBlbWVyZ2VudCBwaGVub3R5cGVzLCB0aGF0IHdpbGwgZmluZCBHIHggTSB0ZWNobm9sb2dpZXMgaW1wcm92aW5nIGZvb2Qgc2VjdXJpdHkgYW5kIG51dHJpdGlvbiBzdXN0YWluYWJseS4gRXhhbXBsZXMgaW5jbHVkZSBjb21iaW5pbmcgRSBhbmQgZ2Vub21pYyBwcmVkaWN0aW9ucyB1c2luZyBjcm9wIG1vZGVscyBhbmQgZW1wb3dlcmluZyBkeW5hbWljICBnZW5lLXRvLXBoZW5vdHlwZSBwbGF0Zm9ybXMgWzI4Nl0uIDUuNi4yIEFkYXB0aXZlIHRyYWl0cywgaGlnaCB0aHJvdWdocHV0IHBoZW5vdHlwaW5nIGFuZCBwaGVub21pY3MgRGVmaW5pbmcgYWRhcHRpdmUgdHJhaXRzIGlzIGFzIGltcG9ydGFudCBhcyBkZWZpbmluZyB0YXJnZXQgZW52aXJvbm1lbnRzIHdoZW4gZGV2ZWxvcC1pbmcgY2xpbWF0ZS1yZXNpbGllbnQgY3VsdGl2YXJzLiBBZGFwdGF0aW9uIHRvIGhpZ2ggdGVtcGVyYXR1cmVzLCBlbGV2YXRlZCBDTzIsIGRyb3VnaHQsIGhlYXQsIHNhbGluaXR5LCBhbmQgd2F0ZXJsb2dnaW5nIHN0cmVzcyB0cmFpdHMgaXMgYWxyZWFkeSBwYXJ0IG9mIGJyZWVkaW5nIHByb2dyYW1zLiBFbXBoYS1zaXMgc2hvdWxkIGJlIGdpdmVuIHRvIHdoaWNoIHRyYWl0cyBoYXZlIHRoZSBiaWdnZXN0IGluZmx1ZW5jZSBhbmQgZmluZGluZyBzcGVjaWZpYyB0cmFpdHMgIG9uIGEgY2FzZS1ieS1jYXNlIGJhc2lzIFsyOTBdLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MjQsImJib3giOnsidG9wIjo3Ni4yNDI3MjE1NTc2MTcxOSwibGVmdCI6MTkuNzYxODkyMzE4NzI1NTg2LCJ3aWR0aCI6NjAuNDc2MTc3MjE1NTc2MTcsImhlaWdodCI6MS44MzEwOTA4MDc5MTQ3MzM5fX0seyJwYWdlTnVtYmVyIjoyNCwiYmJveCI6eyJ0b3AiOjc4LjAyNTUwNTA2NTkxNzk3LCJsZWZ0IjoxOS43NjE4OTIzMTg3MjU1ODYsIndpZHRoIjo2MC44NjQwMDYwNDI0ODA0NywiaGVpZ2h0IjoxLjgzMTA5MDgwNzkxNDczMzl9fSx7InBhZ2VOdW1iZXIiOjI0LCJiYm94Ijp7InRvcCI6NzkuODA4Mjk2MjAzNjEzMjgsImxlZnQiOjE5Ljc2MTg5MjMxODcyNTU4Niwid2lkdGgiOjYwLjg2MDA5MjE2MzA4NTk0LCJoZWlnaHQiOjEuODMxMDkwODA3OTE0NzMzOX19LHsicGFnZU51bWJlciI6MjQsImJib3giOnsidG9wIjo4MS41OTEwNzIwODI1MTk1MywibGVmdCI6MTkuNzYxODkyMzE4NzI1NTg2LCJ3aWR0aCI6NjAuODY5NDQxOTg2MDgzOTg0LCJoZWlnaHQiOjEuODMxMDkwODA3OTE0NzMzOX19LHsicGFnZU51bWJlciI6MjQsImJib3giOnsidG9wIjo4My4zNzM4NzA4NDk2MDkzOCwibGVmdCI6MTkuNzYxODkyMzE4NzI1NTg2LCJ3aWR0aCI6NjAuODU2Nzg4NjM1MjUzOTA2LCJoZWlnaHQiOjEuODMxMDkwODA3OTE0NzMzOX19LHsicGFnZU51bWJlciI6MjQsImJib3giOnsidG9wIjo4NS4xNTY2NTQzNTc5MTAxNiwibGVmdCI6MTkuNzYxODkyMzE4NzI1NTg2LCJ3aWR0aCI6NjAuODU5NTIzNzczMTkzMzYsImhlaWdodCI6MS44MzEwOTA4MDc5MTQ3MzM5fX0seyJwYWdlTnVtYmVyIjoyNCwiYmJveCI6eyJ0b3AiOjg2LjkzOTQzNzg2NjIxMDk0LCJsZWZ0IjoxOS43NjE4OTIzMTg3MjU1ODYsIndpZHRoIjo2MC44NjM4NDU4MjUxOTUzMSwiaGVpZ2h0IjoxLjgzMTA5MDgwNzkxNDczMzl9fSx7InBhZ2VOdW1iZXIiOjI0LCJiYm94Ijp7InRvcCI6ODguNzIyMjI5MDAzOTA2MjUsImxlZnQiOjE5Ljc2MTg5MjMxODcyNTU4Niwid2lkdGgiOjYwLjg2Njk3MDA2MjI1NTg2LCJoZWlnaHQiOjEuODMxMDkwODA3OTE0NzMzOX19LHsicGFnZU51bWJlciI6MjQsImJib3giOnsidG9wIjo5MC41MDUwMTI1MTIyMDcwMywibGVmdCI6MTkuNzYxODkyMzE4NzI1NTg2LCJ3aWR0aCI6NjAuODYwMjQ0NzUwOTc2NTYsImhlaWdodCI6MS44MzEwOTA4MDc5MTQ3MzM5fX0seyJwYWdlTnVtYmVyIjoyNSwiYmJveCI6eyJ0b3AiOjEwLjMxNzc1MDkzMDc4NjEzMywibGVmdCI6MTkuNzYxODkyMzE4NzI1NTg2LCJ3aWR0aCI6NjAuODY5MTQ0NDM5Njk3MjY2LCJoZWlnaHQiOjUuMzk2NjY1NTczMTIwMTE3fX0seyJwYWdlTnVtYmVyIjoyNSwiYmJveCI6eyJ0b3AiOjE3LjQ5NTIxMjU1NDkzMTY0LCJsZWZ0IjoxOS43NjE4OTIzMTg3MjU1ODYsIndpZHRoIjo0My40MjA4Njc5MTk5MjE4NzUsImhlaWdodCI6MS40MTY0NTMwMDM4ODMzNjE4fX0seyJwYWdlTnVtYmVyIjoyNSwiYmJveCI6eyJ0b3AiOjE5LjIzMTY4NzU0NTc3NjM2NywibGVmdCI6MTkuNzYxODkyMzE4NzI1NTg2LCJ3aWR0aCI6NjAuNDc2MTU4MTQyMDg5ODQ0LCJoZWlnaHQiOjEuODMxMDkyODM0NDcyNjU2Mn19LHsicGFnZU51bWJlciI6MjUsImJib3giOnsidG9wIjoyMS4wMTQ0NzY3NzYxMjMwNDcsImxlZnQiOjE5Ljc2MTg5MjMxODcyNTU4Niwid2lkdGgiOjYwLjg2MzE0NzczNTU5NTcsImhlaWdodCI6MS45NDIzMzU0ODY0MTIwNDgzfX0seyJwYWdlTnVtYmVyIjoyNSwiYmJveCI6eyJ0b3AiOjIyLjc5NzI2MDI4NDQyMzgyOCwibGVmdCI6MTkuNzYxODkyMzE4NzI1NTg2LCJ3aWR0aCI6NjAuNDc2MTU4MTQyMDg5ODQ0LCJoZWlnaHQiOjEuODMxMDkyODM0NDcyNjU2Mn19LHsicGFnZU51bWJlciI6MjUsImJib3giOnsidG9wIjoyNC41ODAwMzQyNTU5ODE0NDUsImxlZnQiOjE5Ljc2MTg5MjMxODcyNTU4Niwid2lkdGgiOjYwLjg2NDAyMTMwMTI2OTUzLCJoZWlnaHQiOjEuODMxMDkyODM0NDcyNjU2Mn19LHsicGFnZU51bWJlciI6MjUsImJib3giOnsidG9wIjoyNi4zNjI4MzMwMjMwNzEyOSwibGVmdCI6MTkuNzYxODkyMzE4NzI1NTg2LCJ3aWR0aCI6NjAuODU4NjM4NzYzNDI3NzM0LCJoZWlnaHQiOjEuODMxMDkyODM0NDcyNjU2Mn19XX0sImNodW5rSWQiOiI2ZDhkZWNjYi1hMTUzLTQ5NzQtYTA3Ny0wNzM2MWE4YzFiZDMiLCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIyNSJ9fV0%3D.

### Policy and Regulatory Frameworks

Effective policy and regulatory frameworks are crucial for incentivizing the adoption of advanced irrigation technologies, facilitating water resource reallocation, and ensuring sustainable agricultural practices (Borin, 2024; Chai et al., 2015)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1hdXJpemlvIiwiZmFtaWx5IjoiQm9yaW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMDM4LTU2MjUiXSwiaXNibiI6W10sImRvaSI6IjEwLjM2MjUzL2lqYW0tMjMyMyIsIm9wZW5hbGV4IjoiVzQzOTEyMjg5NjIiLCJvcGVuQWNjZXNzSWQiOiIxMzcxNzc2MTg4NzA3MDU1MDgyNyJ9LCJ0aXRsZSI6IkEgd2lzZSBpcnJpZ2F0aW9uIHRvIGNvbnRyaWJ1dGUgdG8gaW50ZWdyYXRlZCB3YXRlciByZXNvdXJjZSBtYW5hZ2VtZW50IiwiYWJzdHJhY3QiOiJJcnJpZ2F0ZWQgYWdyaWN1bHR1cmUgYWNjb3VudHMgZm9yIGFib3V0IDIwJSBvZiBjdWx0aXZhdGVkIGxhbmRzIHdvcmxkd2lkZSB3aGlsZSBjdXJyZW50bHkgZ2VuZXJhdGluZyBhbiBlc3RpbWF0ZWQgNDAlIG9mIGNyb3AgcHJvZHVjdGlvbiwgYW5kIHR3by10aGlyZHMgb2YgZnV0dXJlIGdhaW5zIGluIGNyb3AgcHJvZHVjdGlvbiBhcmUgZXhwZWN0ZWQgdG8gY29tZSBmcm9tIGlycmlnYXRlZCBsYW5kcy4gVGhlcmVmb3JlLCBpcnJpZ2F0aW9uIGlzIHN0cmF0ZWdpYyB0byBlbnN1cmUgZm9vZCBmb3IgdGhlIHdvcmxk4oCZcyBpbmNyZWFzaW5nIHBvcHVsYXRpb24gYW5kIHNsb3cgZG93biB0aGUgcGFjZSBvZiBkZWZvcmVzdGF0aW9uLiBJcnJpZ2F0ZWQgYWdyaWN1bHR1cmUgYWxzbyBhY2NvdW50cyBmb3IgbW9yZSB0aGFuIDcwJSBvZiBnbG9iYWwgd2F0ZXIgd2l0aGRyYXdhbHMsIGFuZCB0aGUgd2F5IGFncmljdWx0dXJlIHVzZXMgZnJlc2h3YXRlciBpcyBjcnVjaWFsIGZvciBlbnN1cmluZyBhdmFpbGFiaWxpdHkgZm9yIG90aGVyIGFjdGl2aXRpZXMgYW5kIHByZXNlcnZpbmcgd2F0ZXItcmVsYXRlZCBlY29zeXN0ZW1zLiBUbyBnaXZlIGFuIGFncmktZW52aXJvbm1lbnRhbCBpbnRlcnByZXRhdGlvbiBvZiB0aGUgd2F0ZXIgZmx1eGVzIGludm9sdmVkIGluIGlycmlnYXRpb24sIHRoZSB0b3RhbCBhbW91bnQgb2Ygd2F0ZXIgd2l0aGRyYXduIGZyb20gYSBzb3VyY2UgaXMgY2FsbGVkIHdoaXRlIHdhdGVyIChXKSBhbmQgc3ViZGl2aWRlZCBhcyBmb2xsb3dzOiBnb2xkIHdhdGVyIChHKSBpcyB0aGUgYW1vdW50IHRoYXQgaXMgYWN0dWFsbHkgdXNlZCBieSB0aGUgY3JvcHMuIFRoaXMgZnJhY3Rpb24gcmVwcmVzZW50cyB0aGUgdWx0aW1hdGUgZ29hbCBvZiBpcnJpZ2F0aW9uLCB3aGljaCBpcyB0byBpbmNyZWFzZSBjcm9wIHByb2R1Y3Rpdml0eSBhbmQgZm9vZCBhdmFpbGFiaWxpdHkuIFdlIGNhbiBhbHNvIGNvbnNpZGVyIHRoaXMgYW1vdW50IGFzIHRoZSBpcnJpZ2F0aW9uIHdhdGVyIGZvciBjcm9wcy4gVGhlIGdvbGQgY29sb3IgbWVhbnMgYW4gaW5jb21lIGZvciB0aGUgZmFybTsgZW1lcmFsZCB3YXRlciAoRSkgaXMgdGhlIGFtb3VudCBvZiB3YXRlciB3aXRoZHJhd24gZm9yIGlycmlnYXRpb24gdGhhdCBkb2VzIG5vdCByZWFjaCB0aGUgY3JvcHMgYnV0IHByb3ZpZGVzIGVjb3N5c3RlbSBzZXJ2aWNlcywgaS5lLiwgdGhlIGJlbmVmaXRzIHRoYXQgcGVvcGxlIGdldCBmcm9tIGVjb3N5c3RlbXMuIFRoaXMgYW1vdW50IG9mIHdhdGVyIGlzIHVzZWZ1bCBmb3IgdGhlIGNvbW11bml0eSBhbmQgY2FuIGJlIGlkZW50aWZpZWQgYXMgaXJyaWdhdGlvbiBvZiB0aGUgdGVycml0b3J5OyByZWQgd2F0ZXIgKFIpIGlzIGxvc3NlcywgaS5lLiwgdGhlIGZyYWN0aW9uIG9mIHdhdGVyIHRoYXQgaGFzIHRvIGJlIHJlZHVjZWQgYXMgbXVjaCBhcyBwb3NzaWJsZSBpZiBub3QgZWxpbWluYXRlZC4gSXQgaXMgYW4gZWNvbm9taWMsIHNvY2lhbCBhbmQgZW52aXJvbm1lbnRhbCBjb3N0OyB0aGUgcmVkIGNvbG9yIGlzIGFzc29jaWF0ZWQgd2l0aCB0aGUgcmVkIHRyYWZmaWMgbGlnaHQgdGhhdCBtZWFucyBTVE9QISBUaGUgcXVhbnRpZmljYXRpb24gb2YgVyBoYXMgdG8gYmUgcHJlY2VkZWQgYnkgYSBjYXJlZnVsIGV2YWx1YXRpb24gb2YgdGhlIGJlbmVmaXRzIGFjaGlldmFibGUgd2l0aCBpcnJpZ2F0aW9uIGluIHJlbGF0aW9uIHRvIHRoZSBlbnZpcm9ubWVudGFsIGNoYXJhY3RlcmlzdGljcyBvZiB0aGUgY3VsdGl2YXRpb24gYXJlYS4gU3RyYXRlZ2ljIGlzc3VlcyBhdCB0aGlzIHN0YWdlIGFyZSB0aGUgY2hvaWNlIGJldHdlZW4gZnVsbCBvciBkZWZpY2l0IGlycmlnYXRpb24gYW5kIHRoZSBpbmRpdmlkdWF0aW9uIG9mIG9wcG9ydHVuaXRpZXMgdG8gZXhwbG9yYXRpb24gb2Ygb3Bwb3J0dW5pdGllcyB0byBpbmNyZWFzZSB0aGUgYXZhaWxhYmlsaXR5IG9mIHdhdGVyIHJlc291cmNlcyBieSB1c2luZyBub24tY29udmVudGlvbmFsIHNvdXJjZXMuIFRoZSBxdWFudGlmaWNhdGlvbiBvZiBHIGFuZCBFIGZsdXhlcyBpcyBuZWNlc3NhcnkgdG8gcmVkdWNlIGxvc3Nlcywgd2hpbGUgc3RyYXRlZ2llcy90ZWNobmlxdWVzIHVzZWZ1bCB0byByZWR1Y2UgaXJyaWdhdGlvbiBuZWVkcyBhdCB0aGUgZmllbGQgbGV2ZWwgYXJlIGtleSBmYWN0b3JzIHRvIGFsbG9jYXRlIGlycmlnYXRpb24gd2F0ZXIgd2l0aGluIHRoZSBmcmFtZXdvcmsgb2YgYW4gaW50ZWdyYXRlZCBhbmQgc3VzdGFpbmFibGUgbWFuYWdlbWVudCBvZiB3YXRlciBhaW1lZCBhdCB0dXJuaW5nIHVzZXIgY29uZmxpY3RzIGludG8gc3luZXJnaWVzLiBUaGlzIGltcGxpZXMgbXVsdGlwbGUgc3ViamVjdHMgYW5kIGFjdG9ycyBpbiBhIG11bHRpLWRpc2NpcGxpbmFyeSBhcHByb2FjaC4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMzYyNTMvaWphbS0yMzIzIiwicmVzb3VyY2VVcmwiOiJodHRwczovL3JpdmlzdGUuZnVwcmVzcy5uZXQvaW5kZXgucGhwL0lKQW0vYXJ0aWNsZS9kb3dubG9hZC8yMzIzLzE2MjEiLCJpc3N1ZWREYXRlIjoiMjAyNC0wMS0yMCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6Ikl0YWxpYW4gSm91cm5hbCBvZiBBZ3JvbWV0ZW9yb2xvZ3kiLCJpc3N1ZSI6MiwiaW1wYWN0RmFjdG9yIjoiMS4wMCJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjUsImxhc3RQYWdlIjoxOX19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJGaXJlbnplIFVuaXZlcnNpdHkgUHJlc3MifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjM5LCJjaXRlZEJ5Q291bnQiOjJ9fSwicXVvdGUiOiJQcm9taXNpbmcgcmVzdWx0cyBhcmUgIHJlbGF0ZWQgdG8gdGhlIGF2YWlsYWJpbGl0eSBvZiBkcm91Z2h0LXRvbGVyYW50IG1haXplIHZhcmktZXRpZXMgdGhhdCB5aWVsZCBtb3JlIGluIGRyb3VnaHQtc3RyZXNzZWQgZW52aXJvbm1lbnRzICB3aXRoIG5vIHBlbmFsdHkgaW4gbm9uLXN0cmVzc2VkIGVudmlyb25tZW50cyAoQWRlZSAgZXQgYWwuLCAyMDE2KSwgYW5kIHJlcXVpcmUgbGVzcyB3YXRlciB0byBtYXhpbWl6ZSBncmFpbiAgeWllbGQgYXMgY29tcGFyZWQgdG8gdGhlIGNvbnZlbnRpb25hbCBoeWJyaWQgKE1vdW5jZSAgZXQgYWwuLCAyMDE1KS4gVGhlcmVmb3JlLCBjYXJlZnVsIHNlbGVjdGlvbiBvZiBoeWJyaWRzIGNhbiAgaW5jcmVhc2UgY29ybiB5aWVsZCBhbmQgV1VFIHVuZGVyIHdhdGVyLWxpbWl0ZWQgY29uLWRpdGlvbnMgKEhhbyBldCBhbC4sIDIwMTUpLiBUaGUgYWRlcXVhdGUgY2hvaWNlIG9mIGNyb3AgIGFuZCB2YXJpZXR5IGhhcyB0byBiZSBhY2NvbXBhbmllZCBieSBhbGwgdGhlIG90aGVyICBvcHRpb25zIHBlcm1pdHRpbmcgdG8gaW5jcmVhc2UgdGhlIHByb3BvcnRpb24gb2YgZXZhcG8tdHJhbnNwaXJhdGlvbiBmbG93aW5nIGFzIHByb2R1Y3RpdmUgdHJhbnNwaXJhdGlvbiBhcyB0byAgb2J0YWluIOKAnG1vcmUgY3JvcCBwZXIgZHJvcOKAnS4gQWRlcXVhdGUgdGltaW5nIGFuZCBzcGFjLWluZyBvZiBzb3dpbmcsIHdlZWQgYW5kIHBlc3QgY29udHJvbCwgbXVsY2hpbmcsIGFyZSBvbmx5ICBzb21lIGV4YW1wbGVzIG9mIHRoZSB3aWRlIHJhbmdlIG9mIHRoZSBvcHBvcnR1bml0aWVzICB0aGF0IGNhbiBiZSBhZG9wdGVkLiA1LiBDT05DTFVESU5HIFJFTUFSS1MgSXJyaWdhdGlvbiBpcyBpbmRpc3BlbnNhYmxlIHRvIGFjaGlldmUgdGhlIFplcm8gaHVuLWdlciB0YXJnZXQgb2YgdGhlIDIwMzAgVU4gQWdlbmRhIGJlY2F1c2UgaXQgaXMgYSBwb3dlci1mdWwgdG9vbCBmb3Igc3VzdGFpbmFibGUgaW50ZW5zaWZpY2F0aW9uLCBhbmQgYWltcyB0byBwcm8tZHVjZSBtb3JlIHBlciBzdXJmYWNlIHVuaXQuIFRhY2tsaW5nIHRoZSBpc3N1ZSBvZiBlbnN1cmluZyBtb3JlIG51dHJpdGlvdXMgZm9vZCAgZm9yIGEgZ3Jvd2luZyBwb3B1bGF0aW9uLCB0aGUgaGlnaGVyIHByb2R1Y3Rpdml0eSBvZiBpcnJpZ2F0ZWQgbGFuZHMgY2FuIHNsb3cgZG93biB0aGUgcGFjZSBvZiBkZWZvcmVzdGF0aW9uLCBoZW5jZSDigJxtb3JlIGlycmlnYXRpb24sIGxlc3MgZGVmb3Jlc3RhdGlvbuKAnS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo1MS4wNDU2OTI0NDM4NDc2NTYsImxlZnQiOjkuNTIzODAzNzEwOTM3NSwid2lkdGgiOjM5LjY3MjUzMTEyNzkyOTY5LCJoZWlnaHQiOjEuNzYyNTg1NzU5MTYyOTAyOH19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjUyLjYxMzU5Nzg2OTg3MzA1LCJsZWZ0Ijo5LjUyMzgwMzcxMDkzNzUsIndpZHRoIjozOS4yODU5ODAyMjQ2MDkzNzUsImhlaWdodCI6MS43NjI1ODU3NTkxNjI5MDI4fX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NTQuMTgxNDk5NDgxMjAxMTcsImxlZnQiOjkuNTIzODAzNzEwOTM3NSwid2lkdGgiOjM5LjY2MTI1ODY5NzUwOTc2NiwiaGVpZ2h0IjoxLjc2MjU4NTc1OTE2MjkwMjh9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo1NS43NDkzOTcyNzc4MzIwMywibGVmdCI6OS41MjM4MDM3MTA5Mzc1LCJ3aWR0aCI6MzkuNjY3MTQ4NTkwMDg3ODksImhlaWdodCI6MS43NjI1ODU3NTkxNjI5MDI4fX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NTcuMzE3MzA2NTE4NTU0NjksImxlZnQiOjkuNTIzODAzNzEwOTM3NSwid2lkdGgiOjM5LjY3MzM1NTEwMjUzOTA2LCJoZWlnaHQiOjEuNzYyNTg1NzU5MTYyOTAyOH19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjU4Ljg4NTIwNDMxNTE4NTU1LCJsZWZ0Ijo5LjUyMzgwMzcxMDkzNzUsIndpZHRoIjozOS42Njc5OTkyNjc1NzgxMjUsImhlaWdodCI6MS43NjI1ODU3NTkxNjI5MDI4fX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NjAuNDUzMTA1OTI2NTEzNjcsImxlZnQiOjkuNTIzODAzNzEwOTM3NSwid2lkdGgiOjM5LjY2NDI5MTM4MTgzNTk0LCJoZWlnaHQiOjEuNzYyNTg1NzU5MTYyOTAyOH19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjYyLjAyMTAwMzcyMzE0NDUzLCJsZWZ0Ijo5LjUyMzgwMzcxMDkzNzUsIndpZHRoIjozOS4yODU5ODAyMjQ2MDkzNzUsImhlaWdodCI6MS43NjI1ODU3NTkxNjI5MDI4fX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NjMuNTg4OTEyOTYzODY3MTksImxlZnQiOjkuNTIzODAzNzEwOTM3NSwid2lkdGgiOjM5LjY3MDY2NTc0MDk2NjgsImhlaWdodCI6MS43NjI1ODU3NTkxNjI5MDI4fX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NjUuMTU2ODE0NTc1MTk1MzEsImxlZnQiOjkuNTIzODAzNzEwOTM3NSwid2lkdGgiOjM5LjY2NjMwNTU0MTk5MjE5LCJoZWlnaHQiOjEuNzYyNTg1NzU5MTYyOTAyOH19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjY2LjcyNDcxNjE4NjUyMzQ0LCJsZWZ0Ijo5LjUyMzgwMzcxMDkzNzUsIndpZHRoIjozOS4yODU5ODAyMjQ2MDkzNzUsImhlaWdodCI6MS43NjI1ODE4MjUyNTYzNDc3fX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NjguMjkyNjE3Nzk3ODUxNTYsImxlZnQiOjkuNTIzODAzNzEwOTM3NSwid2lkdGgiOjM5LjY2NjgwOTA4MjAzMTI1LCJoZWlnaHQiOjEuNzYyNTgxODI1MjU2MzQ3N319LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjY5Ljg2MDUyNzAzODU3NDIyLCJsZWZ0Ijo5LjUyMzgwMzcxMDkzNzUsIndpZHRoIjozOS4yODU5ODAyMjQ2MDkzNzUsImhlaWdodCI6MS43NjI1ODE4MjUyNTYzNDc3fX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NzEuNDI4NDIxMDIwNTA3ODEsImxlZnQiOjkuNTIzODAzNzEwOTM3NSwid2lkdGgiOjM5LjY2OTMyMjk2NzUyOTMsImhlaWdodCI6MS43NjI1ODE4MjUyNTYzNDc3fX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NzIuOTk2MzIyNjMxODM1OTQsImxlZnQiOjkuNTIzODAzNzEwOTM3NSwid2lkdGgiOjM5LjY2NjQ4NDgzMjc2MzY3LCJoZWlnaHQiOjEuNzYyNTgxODI1MjU2MzQ3N319LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjc0LjU2NDIyNDI0MzE2NDA2LCJsZWZ0Ijo5LjUyMzgwMzcxMDkzNzUsIndpZHRoIjoxMy45ODU5NDk1MTYyOTYzODcsImhlaWdodCI6MS43NjI1ODE4MjUyNTYzNDc3fX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NzkuMjUzOTM2NzY3NTc4MTIsImxlZnQiOjE5LjE2NDY5MDAxNzcwMDE5NSwid2lkdGgiOjIwLjAwMjk5NDUzNzM1MzUxNiwiaGVpZ2h0IjoxLjY3NDQ1MjY2MjQ2Nzk1NjV9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo4Mi4zMjUxMzQyNzczNDM3NSwibGVmdCI6OS41MjM0NDk4OTc3NjYxMTMsIndpZHRoIjozOS42Njc0ODA0Njg3NSwiaGVpZ2h0Ijo5LjYwMjA5MTc4OTI0NTYwNX19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjkuMjg3MDM0OTg4NDAzMzIsImxlZnQiOjUxLjE4OTgzODQwOTQyMzgzLCJ3aWR0aCI6MzkuNjY2NjE4MzQ3MTY3OTcsImhlaWdodCI6MS43NjI1ODI3Nzg5MzA2NjR9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjoxMC44NTQ5MzY1OTk3MzE0NDUsImxlZnQiOjUxLjE4OTgzODQwOTQyMzgzLCJ3aWR0aCI6MzkuNjYzODE4MzU5Mzc1LCJoZWlnaHQiOjEuNzYyNTgyNzc4OTMwNjY0fX1dfSwiY2h1bmtJZCI6ImUxNTg4OGM0LWYwMmUtNGYyYi1iYzE2LTZjNTE0MzY2NzlhZSIsImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEyIn19LHsib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjMwLCJzdWJ0eXBlIjp7fSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUWlhbmciLCJmYW1pbHkiOiJDaGFpIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllhbnRhaSIsImZhbWlseSI6IkdhbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJDYWkiLCJmYW1pbHkiOiJaaGFvIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikh1aS1saWFuIiwiZmFtaWx5IjoiWHUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUmVhZ2FuIiwiZmFtaWx5IjoiV2Fza29tIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllpbmluZyIsImZhbWlseSI6Ik5pdSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLYWRhbWJvdCBILiBNLiIsImZhbWlseSI6IlNpZGRpcXVlIn19XSwiaWRzIjp7Imlzc24iOlsiMTc3My0wMTU1IiwiMTc3NC0wNzQ2Il0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDA3L3MxMzU5My0wMTUtMDMzOC02IiwibWFnIjoiMjIyMzk2MTI4MyIsIm9wZW5hbGV4IjoiVzIyMjM5NjEyODMiLCJvcGVuQWNjZXNzSWQiOiI0NjQwNDc1Mjk5MjE5NTgxMDEifSwidGl0bGUiOiJSZWd1bGF0ZWQgZGVmaWNpdCBpcnJpZ2F0aW9uIGZvciBjcm9wIHByb2R1Y3Rpb24gdW5kZXIgZHJvdWdodCBzdHJlc3MuIEEgcmV2aWV3IiwiYWJzdHJhY3QiOiJBZ3JpY3VsdHVyZSBjb25zdW1lcyBtb3JlIHRoYW4gdHdvIHRoaXJkcyBvZiB0aGUgdG90YWwgZnJlc2h3YXRlciBvZiB0aGUgcGxhbmV0LiBUaGlzIGlzc3VlIGNhdXNlcyBzdWJzdGFudGlhbCBjb25mbGljdCBpbiBmcmVzaHdhdGVyIGFsbG9jYXRpb24gYmV0d2VlbiBhZ3JpY3VsdHVyZSBhbmQgb3RoZXIgZWNvbm9taWMgc2VjdG9ycy4gUmVndWxhdGVkIGRlZmljaXQgaXJyaWdhdGlvbiAoUkRJKSBpcyBrZXkgdGVjaG5vbG9neSBiZWNhdXNlIGl0IGhlbHBzIHRvIGltcHJvdmUgd2F0ZXIgdXNlIGVmZmljaWVuY3kuIE5vbmV0aGVsZXNzLCB0aGVyZSBpcyBhIGxhY2sgb2YgdW5kZXJzdGFuZGluZyBvZiB0aGUgbWVjaGFuaXNtcyB3aXRoIHdoaWNoIHBsYW50cyByZXNwb25kIHRvIFJESS4gSW4gcGFydGljdWxhciwgbGl0dGxlIGlzIGtub3duIGFib3V0IGhvdyBSREkgbWlnaHQgaW5jcmVhc2UgY3JvcCBwcm9kdWN0aW9uIHdoaWxlIHJlZHVjaW5nIHRoZSBhbW91bnQgb2YgaXJyaWdhdGlvbiB3YXRlciBpbiByZWFsLXdvcmxkIGFncmljdWx0dXJlLiBJbiB0aGlzIHJldmlldywgd2UgZm91bmQgdGhhdCBSREkgaXMgbGFyZ2VseSBpbXBsZW1lbnRlZCB0aHJvdWdoIHRocmVlIGFwcHJvYWNoZXM6ICgxKSBncm93dGggc3RhZ2UtYmFzZWQgZGVmaWNpdCBpcnJpZ2F0aW9uLCAoMikgcGFydGlhbCByb290LXpvbmUgaXJyaWdhdGlvbiwgYW5kICgzKSBzdWJzdXJmYWNlIGRyaXBwZXIgaXJyaWdhdGlvbi4gQW1vbmcgdGhlc2UsIHBhcnRpYWwgcm9vdC16b25lIGlycmlnYXRpb24gaXMgdGhlIG1vc3QgcG9wdWxhciBhbmQgZWZmZWN0aXZlIGJlY2F1c2UgbWFueSBmaWVsZCBjcm9wcyBhbmQgc29tZSB3b29keSBjcm9wcyBjYW4gc2F2ZSBpcnJpZ2F0aW9uIHdhdGVyIHVwIHRvIDIwIHRvIDMwICUgd2l0aG91dCBvciB3aXRoIGEgbWluaW1hbCBpbXBhY3Qgb24gY3JvcCB5aWVsZC4gSW1wcm92ZWQgd2F0ZXIgdXNlIGVmZmljaWVuY3kgd2l0aCBSREkgaXMgbWFpbmx5IGR1ZSB0byB0aGUgZm9sbG93aW5nOiAoMSkgZW5oYW5jZWQgZ3VhcmQgY2VsbCBzaWduYWwgdHJhbnNkdWN0aW9uIG5ldHdvcmsgdGhhdCBkZWNyZWFzZXMgdHJhbnNwaXJhdGlvbiB3YXRlciBsb3NzLCAoMikgb3B0aW1pemVkIHN0b21hdGFsIGNvbnRyb2wgdGhhdCBpbXByb3ZlcyB0aGUgcGhvdG9zeW50aGVzaXMgdG8gdHJhbnNwaXJhdGlvbiByYXRpbywgYW5kICgzKSBkZWNyZWFzZWQgZXZhcG9yYXRpdmUgc3VyZmFjZSBhcmVhcyB3aXRoIHBhcnRpYWwgcm9vdC16b25lIGlycmlnYXRpb24gdGhhdCByZWR1Y2VzIHNvaWwgZXZhcG9yYXRpb24uIFRoZSBtZWNoYW5pc21zIGludm9sdmVkIGluIHRoZSBwbGFudCByZXNwb25zZSB0byBSREktaW5kdWNlZCB3YXRlciBzdHJlc3MgaW5jbHVkZSB0aGUgbW9ycGhvbG9naWNhbCB0cmFpdHMsIGUuZy4sIGluY3JlYXNlZCByb290IHRvIHNob290IHJhdGlvIGFuZCBpbXByb3ZlZCBudXRyaWVudCB1cHRha2UgYW5kIHJlY292ZXJ5OyBwaHlzaW9sb2dpY2FsIHRyYWl0cywgZS5nLiwgc3RvbWF0YWwgY2xvc3VyZSwgZGVjcmVhc2VkIGxlYWYgcmVzcGlyYXRpb24sIGFuZCBtYWludGFpbmVkIHBob3Rvc3ludGhlc2lzOyBhbmQgYmlvY2hlbWljYWwgdHJhaXRzLCBlLmcuLCBpbmNyZWFzZWQgc2lnbmFsaW5nIG1vbGVjdWxlcyBhbmQgZW5oYW5jZWQgYW50aW94aWRhdGlvbiBlbnp5bWF0aWMgYWN0aXZpdHkuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjEwMDcvczEzNTkzLTAxNS0wMzM4LTYiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vbGluay5zcHJpbmdlci5jb20vY29udGVudC9wZGYvMTAuMTAwNy9zMTM1OTMtMDE1LTAzMzgtNi5wZGYiLCJpc3N1ZWREYXRlIjoiMjAxNS0xMi0xOCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkFncm9ub215IGZvciBTdXN0YWluYWJsZSBEZXZlbG9wbWVudCIsImlzc3VlIjoxLCJ2b2x1bWUiOjM2LCJpbXBhY3RGYWN0b3IiOiI5Ljc2In0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlNwcmluZ2VyIFNjaWVuY2UrQnVzaW5lc3MgTWVkaWEifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjE0OCwiY2l0ZWRCeUNvdW50Ijo1NTN9fSwicXVvdGUiOiJBZGRpdGlvbmFsbHksIHRoZSByZWFsbG9jYXRpb24gb2Ygd2F0ZXIgcmVzb3VyY2VzIGFuZCBtb2RpZmljYXRpb24gb2YgaXJyaWdhdGlvbiBzeXN0ZW1zIG1heSBoYXZlIGEgcm9sZSB0byBwbGF5IGZvciB0aGUgaW1wcm92ZW1lbnQgb2Ygd2F0ZXIgdXNlIGluIGFncmljdWx0dXJlIChIb21heW91bmZhciBldCBhbC4gMjAxNCkuIDcuMiBDaGFsbGVuZ2VzIFRoZSBjZW50cmFsIG5vdmVsdHkgb2YgUkRJIGlzIHRoYXQgaXQgYWxsb3dzIHBsYW50cyB0byBncm93IHVuZGVyIG1pbGQgd2F0ZXIgc3RyZXNzIHRvIGluZHVjZSByb290LXNvdXJjZWQgY2hlbWljYWwgc2lnLW5hbHMgZnJvbSBkcnlpbmcgcm9vdHMgdG8gcmVhY2ggdGhlIHNob290IHdoZXJlIHBoeXNpb2xvZ2ljYWwgYW5kIGJpb2NoZW1pY2FsIHByb2Nlc3NlcyBjYW4gYmUgcmVndWxhdGVkLiBIb3dldmVyLCB0aGUgcGh5c2lvbG9naWNhbCBhbmQgYmlvY2hlbWljYWwgcmVzcG9uc2VzIGFyZSBkaWZmaWN1bHQgdG8gcXVhbi10aWZ5LCBnaXZlbiB0aGF0IHBhcnRpYWwgcm9vdC16b25lIGlycmlnYXRpb24gaXMgdXN1YWxseSBhcHBsaWVkIHRvIHBsYW50cyBpbiBhY2NvcmRhbmNlIHdpdGggYm90aCB0ZW1wb3JhbCBhbmQgc3BhdGlhbCBkcm91Z2h0cy4gSW4gcHJhY3RpY2UsIGFkb3B0aW9uIG9mIHdhdGVyLXNhdmluZyB0ZWNobmlxdWUgcmVxdWlyZXMgc2NpLWVudGlmaWMgdW5kZXJzdGFuZGluZyBvZiB0aGUgbWVjaGFuaXNtcyBpbnZvbHZlZCBpbiB0aGUgcGh5c2lvbG9naWNhbCBhbmQgYmlvY2hlbWljYWwgcHJvY2Vzc2VzLiBBbHNvLCB0aGUgb3V0Y29tZSBvZiB0aGUgd2F0ZXIgc3RyZXNzLWluZHVjZWQgcmVzcG9uc2VzIGlzIGluZmx1ZW5jZWQgYnkgbWFueSBmYWN0b3JzLCBzdWNoIGFzIHRoZSBzdGFnZXMgb2YgY3JvcCBncm93dGggd2hlbiB0aGUgZGVmaWNpdCBpcyBhcHBsaWVkLCB0aGUgaW50ZW5zaXR5IGFuZCBzZXZlcml0eSBvZiBkZWZpY2l0IGltcG9zZWQsIHRoZSBtZXRob2Qgd2l0aCB3aGljaCBkZWZpY2l0IGlycmlnYXRpb24gaXMgYXBwbGllZCwgY3JvcCBzcGVjaWVzIGFuZCBjdWx0aXZhcnMsIGFuZCB0aGVpciByZXNwZWN0aXZlIG5vbi1jcml0aWNhbCBncm93dGggc3RhZ2VzIHdoZW4gdGhlIHN0YWdlLWJhc2VkIGRlZmljaXQgaXJyaWdhdGlvbiBtYXkgYmUgYXBwbGllZC4gVGhlc2UgZmFjdG9ycyBuZWVkIHRvIGJlIGludmVzdGlnYXRlZCBhdCBhIHN5c3RlbSBsZXZlbCB1bmRlciBzcGVjaWZpYyBncm93aW5nIGNvbmRpdGlvbnMuIFJESSBhbmQsIHBhcnRpY3VsYXJseSwgYWx0ZXJuYXRlIHBhcnRpYWwgcm9vdC16b25lIGlycmlnYXRpb24gYXJlIHN0aWxsIHJlbGF0aXZlbHkgbmV3IHRvIG1hbmFnZSBmYXJtIG1hbmFnZXJzLiBUaGlzIHRlY2gtbmlxdWUgaXMgbm90IGVhc3kgdG8gaW1wbGVtZW50IGluIGNyb3Agc3BlY2llcyB3aXRoIGRlbnNlIHBsYW50IHBvcHVsYXRpb24gc3VjaCBhcyBjZXJlYWwgYW5kIG9pbHNlZWQgY3JvcHMuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjY1LjU0MTQ4MTAxODA2NjQsImxlZnQiOjUxLjQzMTg0NjYxODY1MjM0NCwid2lkdGgiOjM5Ljk5NDI4MTc2ODc5ODgzLCJoZWlnaHQiOjEuMzY0MjYyNzAwMDgwODcxNn19LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjo2Ny4xMTg2NDQ3MTQzNTU0NywibGVmdCI6NTEuNDMxODQ2NjE4NjUyMzQ0LCJ3aWR0aCI6MzkuOTk1OTQxMTYyMTA5Mzc1LCJoZWlnaHQiOjEuMzY0MjYyNzAwMDgwODcxNn19LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjo2OC42ODgyNDAwNTEyNjk1MywibGVmdCI6NTEuNDMxODQ2NjE4NjUyMzQ0LCJ3aWR0aCI6MzkuOTkyNTgwNDEzODE4MzYsImhlaWdodCI6MS4zNjQyNjI3MDAwODA4NzE2fX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjcwLjI2NTM5NjExODE2NDA2LCJsZWZ0Ijo1MS40MzE4NDY2MTg2NTIzNDQsIndpZHRoIjoxNy44OTA4OTIwMjg4MDg1OTQsImhlaWdodCI6MS4zNjQyNjI3MDAwODA4NzE2fX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjczLjQwNzEwNDQ5MjE4NzUsImxlZnQiOjUxLjQzMTg0NjYxODY1MjM0NCwid2lkdGgiOjEwLjEwMzU5MTkxODk0NTMxMiwiaGVpZ2h0IjoxLjM3Njg1NzYzODM1OTA2OTh9fSx7InBhZ2VOdW1iZXIiOjEzLCJiYm94Ijp7InRvcCI6NzYuNTY2NDU5NjU1NzYxNzIsImxlZnQiOjUxLjQzMTg0NjYxODY1MjM0NCwid2lkdGgiOjQwLjAwNzY3MTM1NjIwMTE3LCJoZWlnaHQiOjEzLjk1ODgyMDM0MzAxNzU3OH19LHsicGFnZU51bWJlciI6MTQsImJib3giOnsidG9wIjo3LjI4NDg1ODcwMzYxMzI4MSwibGVmdCI6OC41NzE0MTg3NjIyMDcwMzEsIndpZHRoIjo0MC4wMDc2NDA4Mzg2MjMwNSwiaGVpZ2h0IjoxLjM2NDI2MzE3NjkxODAyOTh9fSx7InBhZ2VOdW1iZXIiOjE0LCJiYm94Ijp7InRvcCI6OC44NTQ0NTMwODY4NTMwMjcsImxlZnQiOjguNTcxNDE4NzYyMjA3MDMxLCJ3aWR0aCI6NDAuMDE1OTk1MDI1NjM0NzY2LCJoZWlnaHQiOjEuMzY0MjYzNjUzNzU1MTg4fX0seyJwYWdlTnVtYmVyIjoxNCwiYmJveCI6eyJ0b3AiOjEwLjQzMTYxMTA2MTA5NjE5MSwibGVmdCI6OC41NzE0MTg3NjIyMDcwMzEsIndpZHRoIjo0MC4wMDc2NDA4Mzg2MjMwNSwiaGVpZ2h0IjoxLjM2NDI2MzY1Mzc1NTE4OH19LHsicGFnZU51bWJlciI6MTQsImJib3giOnsidG9wIjoxMi4wMDg3NjE0MDU5NDQ4MjQsImxlZnQiOjguNTcxNDE4NzYyMjA3MDMxLCJ3aWR0aCI6NDAuMDA0MzAyOTc4NTE1NjI1LCJoZWlnaHQiOjEuMzY0MjYzNjUzNzU1MTg4fX0seyJwYWdlTnVtYmVyIjoxNCwiYmJveCI6eyJ0b3AiOjEzLjU3ODM1NTc4OTE4NDU3LCJsZWZ0Ijo4LjU3MTQxODc2MjIwNzAzMSwid2lkdGgiOjM5Ljk5NzU5Njc0MDcyMjY1NiwiaGVpZ2h0IjoxLjM2NDI2MzY1Mzc1NTE4OH19LHsicGFnZU51bWJlciI6MTQsImJib3giOnsidG9wIjoxNS4xNTU1MTM3NjM0Mjc3MzQsImxlZnQiOjguNTcxNDE4NzYyMjA3MDMxLCJ3aWR0aCI6NDAuMDEwOTkzOTU3NTE5NTMsImhlaWdodCI6MS4zNjQyNjM2NTM3NTUxODh9fSx7InBhZ2VOdW1iZXIiOjE0LCJiYm94Ijp7InRvcCI6MTYuNzMyNjY0MTA4Mjc2MzY3LCJsZWZ0Ijo4LjU3MTQxODc2MjIwNzAzMSwid2lkdGgiOjQwLjAxMDk5MDE0MjgyMjI2NiwiaGVpZ2h0IjoxLjM2NDI2MjcwMDA4MDg3MTZ9fSx7InBhZ2VOdW1iZXIiOjE0LCJiYm94Ijp7InRvcCI6MTguMzAyMjU5NDQ1MTkwNDMsImxlZnQiOjguNTcxNDE4NzYyMjA3MDMxLCJ3aWR0aCI6NDAuMDA3NjM3MDIzOTI1NzgsImhlaWdodCI6MS4zNjQyNjI3MDAwODA4NzE2fX0seyJwYWdlTnVtYmVyIjoxNCwiYmJveCI6eyJ0b3AiOjE5Ljg3OTQxNzQxOTQzMzU5NCwibGVmdCI6OC41NzE0MTg3NjIyMDcwMzEsIndpZHRoIjoxMy4wODkzMDQ5MjQwMTEyMywiaGVpZ2h0IjoxLjM2NDI2MjcwMDA4MDg3MTZ9fSx7InBhZ2VOdW1iZXIiOjE0LCJiYm94Ijp7InRvcCI6MjEuNDU2NTY3NzY0MjgyMjI3LCJsZWZ0IjoxMC40NzU5ODc0MzQzODcyMDcsIndpZHRoIjozOC4xMTE0MjczMDcxMjg5MDYsImhlaWdodCI6MS4zNjQyNjI3MDAwODA4NzE2fX0seyJwYWdlTnVtYmVyIjoxNCwiYmJveCI6eyJ0b3AiOjIzLjAyNjE2MzEwMTE5NjI5LCJsZWZ0Ijo4LjU3MTQxODc2MjIwNzAzMSwid2lkdGgiOjM5Ljk5MjU4ODA0MzIxMjg5LCJoZWlnaHQiOjEuMzY0MjYyNzAwMDgwODcxNn19LHsicGFnZU51bWJlciI6MTQsImJib3giOnsidG9wIjoyNC42MDMzMjEwNzU0Mzk0NTMsImxlZnQiOjguNTcxNDE4NzYyMjA3MDMxLCJ3aWR0aCI6NDAuMDAyNjEzMDY3NjI2OTUsImhlaWdodCI6MS4zNjQyNjI3MDAwODA4NzE2fX0seyJwYWdlTnVtYmVyIjoxNCwiYmJveCI6eyJ0b3AiOjI2LjE4MDQ2OTUxMjkzOTQ1MywibGVmdCI6OC41NzE0MTg3NjIyMDcwMzEsIndpZHRoIjozOS45OTU5MjIwODg2MjMwNSwiaGVpZ2h0IjoxLjM2NDI2MjcwMDA4MDg3MTZ9fV19LCJjaHVua0lkIjoiNzFkNmMwYWMtODg1ZS00YWY0LTgxNDItZTlhZDgxNzFkYzg0IiwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiMTQifX1d. These frameworks often include subsidies for technology adoption, water pricing mechanisms, and mandates for efficient irrigation systems, which collectively drive the transition towards more resilient and water-efficient agricultural landscapes. Moreover, international collaborations and open-access initiatives are essential to democratize advanced biotechnologies, particularly for smallholder farmers in developing regions (Xie, 2025)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjMwLCJzdWJ0eXBlIjp7fSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQ2hlbiIsImZhbWlseSI6IlhpZSJ9fV0sImlkcyI6eyJpc3NuIjpbIjE2NjQtNDYyWCJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMzM4OS9mcGxzLjIwMjUuMTY3NTY1NyIsInBtaWQiOiI0MTAzNjM5MyIsIm9wZW5hbGV4IjoiVzQ0MTQyNDgzMDciLCJvcGVuQWNjZXNzSWQiOiIxNzA5NTc1MjI3OTQ5ODE4NDMxIn0sInRpdGxlIjoiVGhlIHJvbGUgb2YgbW9kZXJuIGFncmljdWx0dXJhbCB0ZWNobm9sb2dpZXMgaW4gaW1wcm92aW5nIGFncmljdWx0dXJhbCBwcm9kdWN0aXZpdHkgYW5kIGxhbmQgdXNlIGVmZmljaWVuY3kiLCJhYnN0cmFjdCI6Ik1vZGVybiBhZ3JpY3VsdHVyYWwgdGVjaG5vbG9naWVzIGFyZSBjcnVjaWFsIGZvciBhZGRyZXNzaW5nIGdsb2JhbCBmb29kIHNlY3VyaXR5IGFuZCBlbnZpcm9ubWVudGFsIHN1c3RhaW5hYmlsaXR5IGNoYWxsZW5nZXMgYW1pZHN0IGEgZ3Jvd2luZyBwb3B1bGF0aW9uIGFuZCBjbGltYXRlIGNoYW5nZS4gVGhlc2UgaW5ub3ZhdGlvbnMsIGluY2x1ZGluZyBwcmVjaXNpb24gYWdyaWN1bHR1cmUsIGJpb3RlY2hub2xvZ3ksIHNtYXJ0IGlycmlnYXRpb24sIGF1dG9tYXRpb24sIHZlcnRpY2FsIGZhcm1pbmcsIGFuZCBhcnRpZmljaWFsIGludGVsbGlnZW5jZSAoQUkpLCBzaWduaWZpY2FudGx5IGVuaGFuY2UgcHJvZHVjdGl2aXR5IGFuZCBsYW5kIHVzZSBlZmZpY2llbmN5LiBQcmVjaXNpb24gYWdyaWN1bHR1cmUsIHV0aWxpemluZyBHUFMsIGRyb25lcywgYW5kIElvVCwgaW1wcm92ZXMgeWllbGRzIGJ5IDIw4oCTMzAlIGFuZCBjdXRzIGlucHV0IHdhc3RlIGJ5IDQw4oCTNjAlLiBCaW90ZWNobm9sb2d5LCB3aXRoIENSSVNQUiBhbmQgR01PcywgZGVsaXZlcnMgZHJvdWdodCBhbmQgcGVzdC1yZXNpc3RhbnQgY3JvcHMsIHN0YWJpbGl6aW5nIHlpZWxkcywgYXMgc2VlbiB3aXRoIEJ0IGNvdHRvbiByZWR1Y2luZyBwZXN0aWNpZGUgdXNlIGJ5IDUwJSBpbiBJbmRpYS4gU21hcnQgaXJyaWdhdGlvbiBib29zdHMgd2F0ZXIgZWZmaWNpZW5jeSBieSA0MOKAkzYwJSwgd2hpbGUgYXV0b21hdGlvbiBhbmQgcm9ib3RpY3MgbWl0aWdhdGUgbGFib3Igc2hvcnRhZ2VzIGFuZCByZWR1Y2UgY29zdHMgYnkgMjUlLiBWZXJ0aWNhbCBmYXJtaW5nIGluY3JlYXNlcyB5aWVsZHMgMTDigJMyMCB0aW1lcyB3aXRoIDk1JSBsZXNzIGxhbmQgYW5kIHdhdGVyLCBzdXBwb3J0aW5nIHVyYmFuIGZvb2Qgc2VjdXJpdHkuIEFJIGFuYWx5dGljcyBlbmhhbmNlIGRlY2lzaW9uLW1ha2luZyB3aXRoIG92ZXIgOTAlIGFjY3VyYWN5IGluIGZvcmVjYXN0aW5nIGFuZCByZXNvdXJjZSBhbGxvY2F0aW9uLiBEZXNwaXRlIHRoZXNlIGJlbmVmaXRzLCBoaWdoIGNvc3RzLCB0ZWNobm9sb2dpY2FsIGlsbGl0ZXJhY3ksIGFuZCByZWd1bGF0b3J5IGlzc3VlcyBoaW5kZXIgYWRvcHRpb24sIGVzcGVjaWFsbHkgYW1vbmcgc21hbGxob2xkZXJzLiBQb2xpY3kgc3VwcG9ydCwgcHVibGljLXByaXZhdGUgcGFydG5lcnNoaXBzLCBhbmQgdHJhaW5pbmcgYXJlIHZpdGFsIGZvciBicm9hZGVyIHRlY2hub2xvZ3kgYWNjZXNzIGFuZCBmYWlyIGJlbmVmaXRzLiBJbnRlZ3JhdGluZyByZW5ld2FibGUgZW5lcmd5IGFuZCBjaXJjdWxhciBlY29ub215IHByaW5jaXBsZXMgaW50byBhZ2dyb3RlY2ggcHJlc2VudHMgYSBwYXRoIHRvIHN1c3RhaW5hYmlsaXR5LiBUaGlzIHJldmlldyBoaWdobGlnaHRzIHRoZSB0cmFuc2Zvcm1hdGl2ZSBwb3RlbnRpYWwgb2YgbW9kZXJuIHRlY2hub2xvZ2llcyBmb3Igc3VzdGFpbmFibGUgaW50ZW5zaWZpY2F0aW9uLCBpbmNyZWFzaW5nIHByb2R1Y3Rpdml0eSB3aXRob3V0IGV4cGFuZGluZyBmYXJtbGFuZCwgd2hpbGUgbGVzc2VuaW5nIGVudmlyb25tZW50YWwgaW1wYWN0cy4gSXQgdW5kZXJzY29yZXMgdGhlIG5lZWQgZm9yIGNvb3JkaW5hdGVkIGVmZm9ydHMgdG8gb3ZlcmNvbWUgYWRvcHRpb24gY2hhbGxlbmdlcyBhbmQgaGFybmVzcyB0aGVzZSBpbm5vdmF0aW9ucyBmb3IgZ2xvYmFsIGZvb2Qgc2VjdXJpdHkgYW5kIGNsaW1hdGUgcmVzaWxpZW5jZS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMzM4OS9mcGxzLjIwMjUuMTY3NTY1NyIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cuZnJvbnRpZXJzaW4ub3JnL2pvdXJuYWxzL3BsYW50LXNjaWVuY2UvYXJ0aWNsZXMvMTAuMzM4OS9mcGxzLjIwMjUuMTY3NTY1Ny9wZGYiLCJpc3N1ZWREYXRlIjoiMjAyNS0wOS0xNiIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZyb250aWVycyBpbiBQbGFudCBTY2llbmNlIiwidm9sdW1lIjoxNiwiaW1wYWN0RmFjdG9yIjoiNy4wMCJ9LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJGcm9udGllcnMgTWVkaWEifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjg4LCJjaXRlZEJ5Q291bnQiOjB9fSwicXVvdGUiOiJTeW50aGV0aWMgYmlvbG9neSBpcyBhbHNvIGNvbnRyaWJ1dGluZywgd2l0aCBtaWNyb2JpYWwgY29uc29ydGlhIGVuZ2luZWVyZWQgdG8gZW5oYW5jZSBzb2lsIGhlYWx0aCBhbmQgY3JvcCByZXNpbGllbmNlIChMYXdzb24gZXQgYWwuLCAyMDE5KS4gRGVzcGl0ZSB0aGVzZSBhZHZhbmNlcywgZXF1aXRhYmxlIGFjY2VzcyByZW1haW5zIGEgY2hhbGxlbmdlLCBhcyBzbWFsbGhvbGRlciBmYXJtZXJzIGluIGRldmVsb3BpbmcgcmVnaW9ucyBvZnRlbiBsYWNrIGFjY2VzcyB0byBwYXRlbnRlZCB0ZWNobm9sb2dpZXMgKFRvZW5uaWVzc2VuIGV0IGFsLiwgMjAwOCkuIEluaXRpYXRpdmVzIGxpa2UgdGhlIE9wZW5DUklTUFIgcHJvamVjdCBhaW0gdG8gZGVtb2NyYXRpemUgZ2VuZS1lZGl0aW5nIHRvb2xzIChDb2hlbiwgMjAyMyksIHdoaWxlIHB1YmxpYy1zZWN0b3IgcGFydG5lcnNoaXBzIChlLmcuLCBDSU1NWVTigJlzIGRyb3VnaHQtdG9sZXJhbnQgbWFpemUpIGVuc3VyZSBhZmZvcmRhYmlsaXR5LiBBcyBjbGltYXRlIGNoYW5nZSBpbnRlbnNpZmllcywgdGhlc2UgYmlvdGVjaG5vbG9naWVzIHdpbGwgYmUgaW5kaXNwZW5zYWJsZSBmb3IgYWNoaWV2aW5nIHN1c3RhaW5hYmxlIGludGVuc2lmaWNhdGlvbiwgYnV0IHRoZWlyIHN1Y2Nlc3MgaGluZ2VzIG9uIGdsb2JhbCBjb2xsYWJvcmF0aW9uLCBldGhpY2FsIGdvdmVybmFuY2UsIGFuZCBpbmNsdXNpdmUgaW5ub3ZhdGlvbi4gMy4zIFNtYXJ0IGlycmlnYXRpb24gJiB3YXRlciBtYW5hZ2VtZW50IFdhdGVyIHNjYXJjaXR5IHBvc2VzIG9uZSBvZiB0aGUgbW9zdCBzaWduaWZpY2FudCBjaGFsbGVuZ2VzIHRvIGdsb2JhbCBhZ3JpY3VsdHVyZSwgd2l0aCA3MCUgb2YgZnJlc2h3YXRlciB3aXRoZHJhd2FscyBhbGxvY2F0ZWQgdG8gaXJyaWdhdGlvbiwgb2Z0ZW4gaW5lZmZpY2llbnRseSAoRkFPLCAyMDIwKS4gU21hcnQgaXJyaWdhdGlvbiBzeXN0ZW1zLCBpbnRlZ3JhdGluZyBJb1QtYmFzZWQgc2Vuc29ycyBhbmQgZHJpcCBpcnJpZ2F0aW9uIHRlY2hub2xvZ2llcywgYXJlIHRyYW5zZm9ybWluZyB3YXRlciBtYW5hZ2VtZW50IGJ5IG9wdGltaXppbmcgdXNhZ2UsIHJlZHVjaW5nIHdhc3RlLCBhbmQgZW5oYW5jaW5nIGNyb3AgeWllbGRzLiBTb2lsIG1vaXN0dXJlIHNlbnNvcnMsIHN1Y2ggYXMgY2FwYWNpdGl2ZSBwcm9iZXMgYW5kIHRlbnNpb21ldGVycywgcHJvdmlkZSByZWFsLXRpbWUgZGF0YSBvbiB3YXRlciBhdmFpbGFiaWxpdHkgYXQgdmFyeWluZyByb290IGRlcHRocywgZW5hYmxpbmcgcHJlY2lzZSBpcnJpZ2F0aW9uIHNjaGVkdWxpbmcuIFRoZXNlIHNlbnNvcnMgdHJhbnNtaXQgZGF0YSB2aWEgTG9SYVdBTiBvciBjZWxsdWxhciBuZXR3b3JrcyB0byBjbG91ZCBwbGF0Zm9ybXMsIHdoZXJlIEFJIGFsZ29yaXRobXMgYW5hbHl6ZSB0cmVuZHMgYW5kIGF1dG9tYXRlIGlycmlnYXRpb24gZGVjaXNpb25zIChHb2FwIGV0IGFsLiwgMjAxOCkuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NTcuMTc1NjQzOTIwODk4NDQsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40OTMxNzU1MDY1OTE4LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjU4LjY2NDIzNDE2MTM3Njk1LCJsZWZ0Ijo4LjMzMTQ0MjgzMjk0Njc3Nywid2lkdGgiOjQwLjQ5Mzk1MzcwNDgzMzk4NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo2MC4xNDYwODAwMTcwODk4NDQsImxlZnQiOjguMzMxNDQxODc5MjcyNDYxLCJ3aWR0aCI6NDAuNDk3NTgxNDgxOTMzNTk0LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjYxLjYzNDY3Nzg4Njk2Mjg5LCJsZWZ0Ijo4LjMzMTQ0MTg3OTI3MjQ2MSwid2lkdGgiOjQwLjQ5NTkwNjgyOTgzMzk4NCwiaGVpZ2h0IjoxLjE4MDUxNjk1ODIzNjY5NDN9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo2My4xMTY1Mjc1NTczNzMwNSwibGVmdCI6OC4zMzE0NDI4MzI5NDY3NzcsIndpZHRoIjo0MC40OTc1Nzc2NjcyMzYzMywiaGVpZ2h0IjoxLjE4MDUxNjk1ODIzNjY5NDN9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo2NC42MDUxMTc3OTc4NTE1NiwibGVmdCI6OC4zMzE0Mzk5NzE5MjM4MjgsIndpZHRoIjo0MC40ODgzNDYwOTk4NTM1MTYsImhlaWdodCI6MS4xODA1MTY5NTgyMzY2OTQzfX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NjYuMDg2OTY3NDY4MjYxNzIsImxlZnQiOjguMzMxNDQxODc5MjcyNDYxLCJ3aWR0aCI6NDAuNDgzMjA3NzAyNjM2NzIsImhlaWdodCI6MS4xODA1MTY5NTgyMzY2OTQzfX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NjcuNTc1NTUzODk0MDQyOTcsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40ODMwNTEzMDAwNDg4MywiaGVpZ2h0IjoxLjE4MDUxNjk1ODIzNjY5NDN9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo2OS4wNTc0MDM1NjQ0NTMxMiwibGVmdCI6OC4zMzE1OTE2MDYxNDAxMzcsIndpZHRoIjo0MC40ODE4NjExMTQ1MDE5NSwiaGVpZ2h0IjoxLjE4MDUxNjk1ODIzNjY5NDN9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo3MC41NDU5OTc2MTk2Mjg5LCJsZWZ0Ijo4LjMzMTU4OTY5ODc5MTUwNCwid2lkdGgiOjQwLjQ4NjA4Mzk4NDM3NSwiaGVpZ2h0IjoxLjE4MDUxNjk1ODIzNjY5NDN9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo3Mi4wMjc4NDcyOTAwMzkwNiwibGVmdCI6OC4zMzE0MzkwMTgyNDk1MTIsIndpZHRoIjozOC4yMTE4NzIxMDA4MzAwOCwiaGVpZ2h0IjoxLjE4MDUxNjk1ODIzNjY5NDN9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo3Ni4xMzk1NDE2MjU5NzY1NiwibGVmdCI6OC4zMzE3NDcwNTUwNTM3MTEsIndpZHRoIjozOC40NTY4MjE0NDE2NTAzOSwiaGVpZ2h0IjoxLjU1ODMzMjIwNDgxODcyNTZ9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo3OS40NTg2MjU3OTM0NTcwMywibGVmdCI6OC4zMzE1OTkyMzU1MzQ2NjgsIndpZHRoIjo0MC41MDk2NTg4MTM0NzY1NiwiaGVpZ2h0IjoxMS41NzM2ODY1OTk3MzE0NDV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo5LjY1NTEzNjEwODM5ODQzOCwibGVmdCI6NTEuMjA5MjE3MDcxNTMzMiwid2lkdGgiOjQwLjQ4NTY0NTI5NDE4OTQ1LCJoZWlnaHQiOjEuMTgwNTE5NTgwODQxMDY0NX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjExLjEzNjk5MTUwMDg1NDQ5MiwibGVmdCI6NTEuMjA5MjE3MDcxNTMzMiwid2lkdGgiOjQwLjQ4MzE5NjI1ODU0NDkyLCJoZWlnaHQiOjEuMTgwNTE5NTgwODQxMDY0NX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjEyLjYyNTU4MzY0ODY4MTY0LCJsZWZ0Ijo1MS4yMDkyMTcwNzE1MzMyLCJ3aWR0aCI6NDAuNDkwNzU2OTg4NTI1MzksImhlaWdodCI6MS4xODA1MTk1ODA4NDEwNjQ1fX1dfSwiY2h1bmtJZCI6IjZhNmQzNjc1LTI2OGEtNDEyMi05NzFmLWU4ZjcwZjljZTQwYiIsImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjYifX1d. Beyond policy, innovative crop breeding plays a pivotal role in bolstering agricultural resilience, particularly through the development of drought-tolerant and water-efficient varieties (Chami & Moujabber, 2016)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkRhbmllbCBFbCIsImZhbWlseSI6IkNoYW1pIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1hcm91biBFbCIsImZhbWlseSI6Ik1vdWphYmJlciJ9fV0sImlkcyI6eyJpc3NuIjpbIjAwMzgtMjM1MyIsIjE5OTYtNzQ4OSJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMTcxNTkvc2Fqcy4yMDE2LzIwMTUwNDU3IiwibWFnIjoiMjUyMzY3OTY5OCIsIm9wZW5hbGV4IjoiVzI1MjM2Nzk2OTgiLCJvcGVuQWNjZXNzSWQiOiIxNTU1ODU3MTcyMDQzNzYyMTg0OSJ9LCJ0aXRsZSI6IkRyb3VnaHQsIGNsaW1hdGUgY2hhbmdlIGFuZCBzdXN0YWluYWJpbGl0eSBvZiB3YXRlciBpbiBhZ3JpY3VsdHVyZTogQSByb2FkbWFwIHRvd2FyZHMgdGhlIE5XUlMyIiwiYWJzdHJhY3QiOiJUaGUgZnJlcXVlbmN5IGFuZCBpbnRlbnNpdHkgb2YgZHJvdWdodCwgZXh0cmVtZSBldmVudHMgYW5kIGhpZ2ggd2luZCB2ZWxvY2l0aWVzIGluIFNvdXRoIEFmcmljYSBhcmUgZXhwZWN0ZWQgdG8gaW5jcmVhc2UgaW4gdGhlIG5leHQgY2VudHVyeSBhcyBhIHJlc3VsdCBvZiB0aGUgY2hhbmdpbmcgY2xpbWF0ZS4gVGhlIE5hdGlvbmFsIFdhdGVyIFJlc291cmNlIFN0cmF0ZWd5IDIgKE5XUlMyKSBoYXMgc2V0IG91dCB0aGUgZ2VuZXJhbCBhbmQgc3RyYXRlZ2ljIGRpcmVjdGlvbnMgZm9yIHdhdGVyIHJlc291cmNlcyBtYW5hZ2VtZW50IGluIHRoZSBjb3VudHJ5IGZvciB0aGUgbmV4dCAyMCB5ZWFycy4gSG93ZXZlciwgdGhlIHN0cmF0ZWd5IGRvZXMgbm90IGRyYXcgYSBmcmFtZXdvcmsgdGFpbG9yZWQgc3BlY2lmaWNhbGx5IGZvciBhZ3JpY3VsdHVyYWwgdXNlLCB3aXRoIHNwZWNpZmljIG1lYXN1cmVzIGFuZCBnb2Fscy4gVGhlcmVmb3JlLCB0byByZWFjaCBzdXN0YWluYWJpbGl0eSBvZiB3YXRlciBpbiBhZ3JpY3VsdHVyZSwgZm91ciBtYWpvciBzdHJhdGVnaWMgZ29hbHMgYXJlIHN1Z2dlc3RlZCwgb24gd2hpY2ggcmVzZWFyY2ggaW5zdGl0dXRpb25zIGNhbiBmb2N1cyBhbmQgcHJvbW90ZSB0aHJvdWdoIGdvb2QgZ292ZXJuYW5jZS4gVGhlIHN0cmF0ZWd5IGVtcGhhc2lzZXM6ICgxKSBjcm9wIHJlc2VhcmNoIHRvIGZpbmQgbmV3IGRyb3VnaHQtYW5kIGhlYXQtIHRvbGVyYW50IGFuZCByZXNpc3RhbnQgYnJlZWRzIGFuZCB2YXJpZXRpZXM7ICgyKSBpbnRlbnNpZmllZCByZXNlYXJjaCBpbiBhZ3JpY3VsdHVyYWwgcHJhY3RpY2VzOyAoMykgaW5jcmVhc2luZyB0aGUgZWZmaWNpZW5jeSBvZiB3YXRlciB1c2Ugd2l0aGluIGFncmljdWx0dXJlOyBhbmQgKDQpIGludGVncmF0aW5nIGFsbCB0aGVzZSBzdHJhdGVnaWMgZ29hbHMgd2l0aGluIGEgc3VzdGFpbmFibGUgcmVzZWFyY2ggZnJhbWV3b3JrLiBGaW5hbGx5LCB0aGUgcmVzZWFyY2ggY2FsbHMgZm9yIHJhcGlkIGFjdGlvbiBhbmQgaW1wbGVtZW50YXRpb24uIFNpZ25pZmljYW5jZTogVGhlIGZyYW1ld29yayBpcyBwcm9wb3NlZCBmb3Igc3Rha2Vob2xkZXJzIGFuZCBwb2xpY3ltYWtlcnMgaW4gaGlnaGVyIGVkdWNhdGlvbiwgYWdyaWN1bHR1cmUgYW5kIHJlc291cmNlcyBtYW5hZ2VtZW50IGluIFNvdXRoIEFmcmljYSBmb3IgbmV3IHJlc2VhcmNoIGhvcml6b25zIGF0IG5hdGlvbmFsIGxldmVsIHRvIGltcHJvdmUgb3ZlcmFsbCBhZ3JpY3VsdHVyYWwgc3VzdGFpbmFiaWxpdHkgYnkgMjAzMCBhcyBzdGlwdWxhdGVkIGJ5IHRoZSBNaWxsZW5uaXVtIERldmVsb3BtZW50IEdvYWxzLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xNzE1OS9zYWpzLjIwMTYvMjAxNTA0NTciLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vd3d3LnNhanMuY28uemEvYXJ0aWNsZS9kb3dubG9hZC8zNjYzLzQ3NjkiLCJpc3N1ZWREYXRlIjoiMjAxNi0wOS0yMyIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IlNvdXRoIEFmcmljYW4gSm91cm5hbCBvZiBTY2llbmNlIiwidm9sdW1lIjoxMTIsImltcGFjdEZhY3RvciI6IjEuNDIifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjo0LCJsYXN0UGFnZSI6NH19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJBY2FkZW15IG9mIFNjaWVuY2Ugb2YgU291dGggQWZyaWNhIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjozOSwiY2l0ZWRCeUNvdW50IjoyNX19LCJxdW90ZSI6IkluIHRoZSBtZWFudGltZSwgY3JvcCBicmVlZGluZyAoZXNwZWNpYWxseSB0aGUgIGNvbnZlbnRpb25hbCBicmVlZGluZyB1c2luZyBtYXJrZXItYXNzaXN0ZWQgc2VsZWN0aW9uKSBpcyBhbiBleHRyZW1lbHkgIHVzZWZ1bCB0ZWNobmlxdWUgZm9yIGJyZWVkaW5nIGNvbXBsZXggdHJhaXRzIChzdWNoIGFzIGRyb3VnaHQgdG9sZXJhbmNlKSAgaW50byBpbXByb3ZlZCB2YXJpZXRpZXMuMjIgVGhlcmUgaXMgYSBkZWFydGggb2Ygc3R1ZGllcyByZXBvcnRlZCBpbiB0aGUgbGl0ZXJhdHVyZSB0byBlbmFibGUgcXVhbnRpLWZpY2F0aW9uIG9mIHdhdGVyIGNvbnN1bXB0aW9uIGFuZCBzYXZpbmdzIGFzIGEgcmVzdWx0IG9mIHN1Y2ggaW1wcm92ZWQgdmFyaWV0aWVzLiBIb3dldmVyLCBhIHN0dWR5IGhhcyBzaG93biB0aGF0IGRyb3VnaHQtdG9sZXJhbnQgYW5kIGRyb3VnaHQtcmVzaXN0YW50IHRyYWl0cyBjb3VsZCBpbXByb3ZlIHRoZSB5aWVsZCBvZiBtYWl6ZSBpbiBBZnJpY2EgYnkgdXAgdG8gMzAlICB1bmRlciBkcm91Z2h0IGNvbmRpdGlvbnMgd2hlbiBjb21wYXJlZCB3aXRoIGNvbW1lcmNpYWwgc2VlZHMuMjMgIEZ1cnRoZXJtb3JlLCBHdXIgYW5kIFphbWlyMjQgc3VjY2Vzc2Z1bGx5IG9idGFpbmVkIDUwJSBoaWdoZXIgeWllbGRzIG9mICB0b21hdG8sIGV2ZW4gdW5kZXIgZHJvdWdodCBjb25kaXRpb25zLCBieSBicmVlZGluZyBjdWx0aXZhdGVkIHRvbWF0b2VzICB3aXRoIHdpbGQgcmVsYXRpdmVzLiBEZXJ5bmcgZXQgYWwuMjUgZm91bmQg4oCTIG9uIGEgZ2xvYmFsIHNjYWxlIOKAkyB0aGF0IGJ5IHRoZSAgMjA4MHMsIGV4dHJlbWUgaGVhdCBzdHJlc3MgYXQgYW50aGVzaXMgY291bGQ6ICgxKSBkb3VibGUgbG9zc2VzIG9mIG1haXplICB5aWVsZCwgKDIpIHJlZHVjZSBwcm9qZWN0ZWQgZ2FpbnMgaW4gc3ByaW5nIHdoZWF0IHlpZWxkIGJ5IGhhbGYsIGFuZCAoMykgIHJlZHVjZSBwcm9qZWN0ZWQgZ2FpbnMgaW4gc295YmVhbiB5aWVsZCBieSBhIHF1YXJ0ZXIuIEFncmljdWx0dXJhbCBwcmFjdGljZXMgQmlvZGl2ZXJzaXR5IGFuZCBoZWFsdGh5IHNvaWwgYXJlIGNlbnRyYWwgdG8gZWNvbG9naWNhbCBhcHByb2FjaGVzIGZvciBtb3JlIGRyb3VnaHQtcmVzaXN0YW50IGFuZCBkcm91Z2h0LXRvbGVyYW50IGZhcm1pbmcgcHJhY3RpY2VzLCB3aGljaCBpcyAgbW9yZSByZXNpbGllbnQgdG8gZXh0cmVtZSBldmVudHMuMjYgUHJhY3RpY2VzIGVuY291cmFnaW5nIGJpb2RpdmVyc2l0eSAgYW5kIHNvaWwgaGVhbHRoIHdvdWxkIGluY3JlYXNlIHRoZSBhdmFpbGFibGUgd2F0ZXIgZm9yIGNyb3BzIGFuZCBpbXByb3ZlICBzdXN0YWluYWJpbGl0eSBvZiBhZ3JpY3VsdHVyYWwgcHJvZHVjdGlvbi4yNyBNYW55IG9mIHRoZXNlIGFncmljdWx0dXJhbCBwcmFjdGljZXMgYXJlIGF2YWlsYWJsZSBhbmQgYXJlIHVzZWQgaW4gZGlmZmVyZW50IHBsYWNlcyBpbiB0aGUgd29ybGQgdG8gbWFuYWdlIGNyb3BzIGFuZCBzb2lscy4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ny4zOTAxNjM0MjE2MzA4NiwibGVmdCI6OS41MjM4MDA4NDk5MTQ1NSwid2lkdGgiOjM5LjE2NjI5MDI4MzIwMzEyNSwiaGVpZ2h0IjoxLjIxMDI3NzkxNTAwMDkxNTV9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0OC42MTI2MTM2Nzc5Nzg1MTYsImxlZnQiOjkuNTIzODAwODQ5OTE0NTUsIndpZHRoIjozOS4xNjI4NzIzMTQ0NTMxMjUsImhlaWdodCI6MS4yMTAyNzc5MTUwMDA5MTU1fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDkuODM1MDUyNDkwMjM0Mzc1LCJsZWZ0Ijo5LjUyMzgwMDg0OTkxNDU1LCJ3aWR0aCI6MzkuMTY2MTQ1MzI0NzA3MDMsImhlaWdodCI6MS4yMTAyNzc5MTUwMDA5MTU1fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTEuMDU3NDk1MTE3MTg3NSwibGVmdCI6OS41MjM4MDA4NDk5MTQ1NSwid2lkdGgiOjEzLjQxNTQyNjI1NDI3MjQ2MSwiaGVpZ2h0IjoxLjIxMDI3NzkxNTAwMDkxNTV9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1Mi45NTMzMzA5OTM2NTIzNDQsImxlZnQiOjkuNTIzODAwODQ5OTE0NTUsIndpZHRoIjozOS4xNjgyODE1NTUxNzU3OCwiaGVpZ2h0IjoxMy40MzQ3MTUyNzA5OTYwOTR9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2Ny4xOTU5NDU3Mzk3NDYxLCJsZWZ0Ijo5LjUyMzgwMDg0OTkxNDU1LCJ3aWR0aCI6MTcuMTU2OTY1MjU1NzM3MzA1LCJoZWlnaHQiOjEuNzEwMDE5NTg4NDcwNDU5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NjkuMTU1Mzk1NTA3ODEyNSwibGVmdCI6OS41MjM4MDA4NDk5MTQ1NSwid2lkdGgiOjM5LjE2NzMwMTE3Nzk3ODUxNiwiaGVpZ2h0Ijo2LjEwMDA0OTk3MjUzNDE4fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NzUuOTQxMDAxODkyMDg5ODQsImxlZnQiOjkuNTIzODAwODQ5OTE0NTUsIndpZHRoIjozOS4xNjIxNDc1MjE5NzI2NTYsImhlaWdodCI6MS4yMTAyNzc5MTUwMDA5MTU1fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NzcuMTYzNDU5Nzc3ODMyMDMsImxlZnQiOjkuNTIzODAwODQ5OTE0NTUsIndpZHRoIjozOS4xNjI3MTU5MTE4NjUyMzQsImhlaWdodCI6MS4yMTAyNzc5MTUwMDA5MTU1fX1dfSwiY2h1bmtJZCI6IjAxZjIxZDQ4LTdlMDEtNDFiMy1hNjIzLTY2YzkxMzU3Yzk4YiIsImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjUifX1d.

## Challenges and Future Directions

### Technological Barriers and Infrastructure Gaps

Advanced irrigation technologies face significant technological barriers, including limited access to real-time sensors, IoT integration, and automated systems in remote or developing regions, where unreliable electricity and poor internet connectivity hinder deployment (Daraz et al., 2025; Xie, 2025)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjMwLCJzdWJ0eXBlIjp7fSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQ2hlbiIsImZhbWlseSI6IlhpZSJ9fV0sImlkcyI6eyJpc3NuIjpbIjE2NjQtNDYyWCJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMzM4OS9mcGxzLjIwMjUuMTY3NTY1NyIsInBtaWQiOiI0MTAzNjM5MyIsIm9wZW5hbGV4IjoiVzQ0MTQyNDgzMDcifSwidGl0bGUiOiJUaGUgcm9sZSBvZiBtb2Rlcm4gYWdyaWN1bHR1cmFsIHRlY2hub2xvZ2llcyBpbiBpbXByb3ZpbmcgYWdyaWN1bHR1cmFsIHByb2R1Y3Rpdml0eSBhbmQgbGFuZCB1c2UgZWZmaWNpZW5jeSIsImFic3RyYWN0IjoiTW9kZXJuIGFncmljdWx0dXJhbCB0ZWNobm9sb2dpZXMgYXJlIGNydWNpYWwgZm9yIGFkZHJlc3NpbmcgZ2xvYmFsIGZvb2Qgc2VjdXJpdHkgYW5kIGVudmlyb25tZW50YWwgc3VzdGFpbmFiaWxpdHkgY2hhbGxlbmdlcyBhbWlkc3QgYSBncm93aW5nIHBvcHVsYXRpb24gYW5kIGNsaW1hdGUgY2hhbmdlLiBUaGVzZSBpbm5vdmF0aW9ucywgaW5jbHVkaW5nIHByZWNpc2lvbiBhZ3JpY3VsdHVyZSwgYmlvdGVjaG5vbG9neSwgc21hcnQgaXJyaWdhdGlvbiwgYXV0b21hdGlvbiwgdmVydGljYWwgZmFybWluZywgYW5kIGFydGlmaWNpYWwgaW50ZWxsaWdlbmNlIChBSSksIHNpZ25pZmljYW50bHkgZW5oYW5jZSBwcm9kdWN0aXZpdHkgYW5kIGxhbmQgdXNlIGVmZmljaWVuY3kuIFByZWNpc2lvbiBhZ3JpY3VsdHVyZSwgdXRpbGl6aW5nIEdQUywgZHJvbmVzLCBhbmQgSW9ULCBpbXByb3ZlcyB5aWVsZHMgYnkgMjDigJMzMCUgYW5kIGN1dHMgaW5wdXQgd2FzdGUgYnkgNDDigJM2MCUuIEJpb3RlY2hub2xvZ3ksIHdpdGggQ1JJU1BSIGFuZCBHTU9zLCBkZWxpdmVycyBkcm91Z2h0IGFuZCBwZXN0LXJlc2lzdGFudCBjcm9wcywgc3RhYmlsaXppbmcgeWllbGRzLCBhcyBzZWVuIHdpdGggQnQgY290dG9uIHJlZHVjaW5nIHBlc3RpY2lkZSB1c2UgYnkgNTAlIGluIEluZGlhLiBTbWFydCBpcnJpZ2F0aW9uIGJvb3N0cyB3YXRlciBlZmZpY2llbmN5IGJ5IDQw4oCTNjAlLCB3aGlsZSBhdXRvbWF0aW9uIGFuZCByb2JvdGljcyBtaXRpZ2F0ZSBsYWJvciBzaG9ydGFnZXMgYW5kIHJlZHVjZSBjb3N0cyBieSAyNSUuIFZlcnRpY2FsIGZhcm1pbmcgaW5jcmVhc2VzIHlpZWxkcyAxMOKAkzIwIHRpbWVzIHdpdGggOTUlIGxlc3MgbGFuZCBhbmQgd2F0ZXIsIHN1cHBvcnRpbmcgdXJiYW4gZm9vZCBzZWN1cml0eS4gQUkgYW5hbHl0aWNzIGVuaGFuY2UgZGVjaXNpb24tbWFraW5nIHdpdGggb3ZlciA5MCUgYWNjdXJhY3kgaW4gZm9yZWNhc3RpbmcgYW5kIHJlc291cmNlIGFsbG9jYXRpb24uIERlc3BpdGUgdGhlc2UgYmVuZWZpdHMsIGhpZ2ggY29zdHMsIHRlY2hub2xvZ2ljYWwgaWxsaXRlcmFjeSwgYW5kIHJlZ3VsYXRvcnkgaXNzdWVzIGhpbmRlciBhZG9wdGlvbiwgZXNwZWNpYWxseSBhbW9uZyBzbWFsbGhvbGRlcnMuIFBvbGljeSBzdXBwb3J0LCBwdWJsaWMtcHJpdmF0ZSBwYXJ0bmVyc2hpcHMsIGFuZCB0cmFpbmluZyBhcmUgdml0YWwgZm9yIGJyb2FkZXIgdGVjaG5vbG9neSBhY2Nlc3MgYW5kIGZhaXIgYmVuZWZpdHMuIEludGVncmF0aW5nIHJlbmV3YWJsZSBlbmVyZ3kgYW5kIGNpcmN1bGFyIGVjb25vbXkgcHJpbmNpcGxlcyBpbnRvIGFnZ3JvdGVjaCBwcmVzZW50cyBhIHBhdGggdG8gc3VzdGFpbmFiaWxpdHkuIFRoaXMgcmV2aWV3IGhpZ2hsaWdodHMgdGhlIHRyYW5zZm9ybWF0aXZlIHBvdGVudGlhbCBvZiBtb2Rlcm4gdGVjaG5vbG9naWVzIGZvciBzdXN0YWluYWJsZSBpbnRlbnNpZmljYXRpb24sIGluY3JlYXNpbmcgcHJvZHVjdGl2aXR5IHdpdGhvdXQgZXhwYW5kaW5nIGZhcm1sYW5kLCB3aGlsZSBsZXNzZW5pbmcgZW52aXJvbm1lbnRhbCBpbXBhY3RzLiBJdCB1bmRlcnNjb3JlcyB0aGUgbmVlZCBmb3IgY29vcmRpbmF0ZWQgZWZmb3J0cyB0byBvdmVyY29tZSBhZG9wdGlvbiBjaGFsbGVuZ2VzIGFuZCBoYXJuZXNzIHRoZXNlIGlubm92YXRpb25zIGZvciBnbG9iYWwgZm9vZCBzZWN1cml0eSBhbmQgY2xpbWF0ZSByZXNpbGllbmNlLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzg5L2ZwbHMuMjAyNS4xNjc1NjU3IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5mcm9udGllcnNpbi5vcmcvam91cm5hbHMvcGxhbnQtc2NpZW5jZS9hcnRpY2xlcy8xMC4zMzg5L2ZwbHMuMjAyNS4xNjc1NjU3L3BkZiIsImlzc3VlZERhdGUiOiIyMDI1LTA5LTE2IiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiRnJvbnRpZXJzIGluIFBsYW50IFNjaWVuY2UiLCJ2b2x1bWUiOjE2LCJpbXBhY3RGYWN0b3IiOiI3LjAwIn0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkZyb250aWVycyBNZWRpYSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6ODgsImNpdGVkQnlDb3VudCI6MH19LCJxdW90ZSI6IlN5bnRoZXRpYyBiaW9sb2d5IGlzIGFsc28gY29udHJpYnV0aW5nLCB3aXRoIG1pY3JvYmlhbCBjb25zb3J0aWEgZW5naW5lZXJlZCB0byBlbmhhbmNlIHNvaWwgaGVhbHRoIGFuZCBjcm9wIHJlc2lsaWVuY2UgKExhd3NvbiBldCBhbC4sIDIwMTkpLiBEZXNwaXRlIHRoZXNlIGFkdmFuY2VzLCBlcXVpdGFibGUgYWNjZXNzIHJlbWFpbnMgYSBjaGFsbGVuZ2UsIGFzIHNtYWxsaG9sZGVyIGZhcm1lcnMgaW4gZGV2ZWxvcGluZyByZWdpb25zIG9mdGVuIGxhY2sgYWNjZXNzIHRvIHBhdGVudGVkIHRlY2hub2xvZ2llcyAoVG9lbm5pZXNzZW4gZXQgYWwuLCAyMDA4KS4gSW5pdGlhdGl2ZXMgbGlrZSB0aGUgT3BlbkNSSVNQUiBwcm9qZWN0IGFpbSB0byBkZW1vY3JhdGl6ZSBnZW5lLWVkaXRpbmcgdG9vbHMgKENvaGVuLCAyMDIzKSwgd2hpbGUgcHVibGljLXNlY3RvciBwYXJ0bmVyc2hpcHMgKGUuZy4sIENJTU1ZVOKAmXMgZHJvdWdodC10b2xlcmFudCBtYWl6ZSkgZW5zdXJlIGFmZm9yZGFiaWxpdHkuIEFzIGNsaW1hdGUgY2hhbmdlIGludGVuc2lmaWVzLCB0aGVzZSBiaW90ZWNobm9sb2dpZXMgd2lsbCBiZSBpbmRpc3BlbnNhYmxlIGZvciBhY2hpZXZpbmcgc3VzdGFpbmFibGUgaW50ZW5zaWZpY2F0aW9uLCBidXQgdGhlaXIgc3VjY2VzcyBoaW5nZXMgb24gZ2xvYmFsIGNvbGxhYm9yYXRpb24sIGV0aGljYWwgZ292ZXJuYW5jZSwgYW5kIGluY2x1c2l2ZSBpbm5vdmF0aW9uLiAzLjMgU21hcnQgaXJyaWdhdGlvbiAmIHdhdGVyIG1hbmFnZW1lbnQgV2F0ZXIgc2NhcmNpdHkgcG9zZXMgb25lIG9mIHRoZSBtb3N0IHNpZ25pZmljYW50IGNoYWxsZW5nZXMgdG8gZ2xvYmFsIGFncmljdWx0dXJlLCB3aXRoIDcwJSBvZiBmcmVzaHdhdGVyIHdpdGhkcmF3YWxzIGFsbG9jYXRlZCB0byBpcnJpZ2F0aW9uLCBvZnRlbiBpbmVmZmljaWVudGx5IChGQU8sIDIwMjApLiBTbWFydCBpcnJpZ2F0aW9uIHN5c3RlbXMsIGludGVncmF0aW5nIElvVC1iYXNlZCBzZW5zb3JzIGFuZCBkcmlwIGlycmlnYXRpb24gdGVjaG5vbG9naWVzLCBhcmUgdHJhbnNmb3JtaW5nIHdhdGVyIG1hbmFnZW1lbnQgYnkgb3B0aW1pemluZyB1c2FnZSwgcmVkdWNpbmcgd2FzdGUsIGFuZCBlbmhhbmNpbmcgY3JvcCB5aWVsZHMuIFNvaWwgbW9pc3R1cmUgc2Vuc29ycywgc3VjaCBhcyBjYXBhY2l0aXZlIHByb2JlcyBhbmQgdGVuc2lvbWV0ZXJzLCBwcm92aWRlIHJlYWwtdGltZSBkYXRhIG9uIHdhdGVyIGF2YWlsYWJpbGl0eSBhdCB2YXJ5aW5nIHJvb3QgZGVwdGhzLCBlbmFibGluZyBwcmVjaXNlIGlycmlnYXRpb24gc2NoZWR1bGluZy4gVGhlc2Ugc2Vuc29ycyB0cmFuc21pdCBkYXRhIHZpYSBMb1JhV0FOIG9yIGNlbGx1bGFyIG5ldHdvcmtzIHRvIGNsb3VkIHBsYXRmb3Jtcywgd2hlcmUgQUkgYWxnb3JpdGhtcyBhbmFseXplIHRyZW5kcyBhbmQgYXV0b21hdGUgaXJyaWdhdGlvbiBkZWNpc2lvbnMgKEdvYXAgZXQgYWwuLCAyMDE4KS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo1Ny4xNzU2NDM5MjA4OTg0NCwibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ5MzE3NTUwNjU5MTgsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NTguNjY0MjM0MTYxMzc2OTUsImxlZnQiOjguMzMxNDQyODMyOTQ2Nzc3LCJ3aWR0aCI6NDAuNDkzOTUzNzA0ODMzOTg0LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjYwLjE0NjA4MDAxNzA4OTg0NCwibGVmdCI6OC4zMzE0NDE4NzkyNzI0NjEsIndpZHRoIjo0MC40OTc1ODE0ODE5MzM1OTQsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NjEuNjM0Njc3ODg2OTYyODksImxlZnQiOjguMzMxNDQxODc5MjcyNDYxLCJ3aWR0aCI6NDAuNDk1OTA2ODI5ODMzOTg0LCJoZWlnaHQiOjEuMTgwNTE2OTU4MjM2Njk0M319LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjYzLjExNjUyNzU1NzM3MzA1LCJsZWZ0Ijo4LjMzMTQ0MjgzMjk0Njc3Nywid2lkdGgiOjQwLjQ5NzU3NzY2NzIzNjMzLCJoZWlnaHQiOjEuMTgwNTE2OTU4MjM2Njk0M319LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjY0LjYwNTExNzc5Nzg1MTU2LCJsZWZ0Ijo4LjMzMTQzOTk3MTkyMzgyOCwid2lkdGgiOjQwLjQ4ODM0NjA5OTg1MzUxNiwiaGVpZ2h0IjoxLjE4MDUxNjk1ODIzNjY5NDN9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo2Ni4wODY5Njc0NjgyNjE3MiwibGVmdCI6OC4zMzE0NDE4NzkyNzI0NjEsIndpZHRoIjo0MC40ODMyMDc3MDI2MzY3MiwiaGVpZ2h0IjoxLjE4MDUxNjk1ODIzNjY5NDN9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo2Ny41NzU1NTM4OTQwNDI5NywibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ4MzA1MTMwMDA0ODgzLCJoZWlnaHQiOjEuMTgwNTE2OTU4MjM2Njk0M319LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjY5LjA1NzQwMzU2NDQ1MzEyLCJsZWZ0Ijo4LjMzMTU5MTYwNjE0MDEzNywid2lkdGgiOjQwLjQ4MTg2MTExNDUwMTk1LCJoZWlnaHQiOjEuMTgwNTE2OTU4MjM2Njk0M319LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjcwLjU0NTk5NzYxOTYyODksImxlZnQiOjguMzMxNTg5Njk4NzkxNTA0LCJ3aWR0aCI6NDAuNDg2MDgzOTg0Mzc1LCJoZWlnaHQiOjEuMTgwNTE2OTU4MjM2Njk0M319LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjcyLjAyNzg0NzI5MDAzOTA2LCJsZWZ0Ijo4LjMzMTQzOTAxODI0OTUxMiwid2lkdGgiOjM4LjIxMTg3MjEwMDgzMDA4LCJoZWlnaHQiOjEuMTgwNTE2OTU4MjM2Njk0M319LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjc2LjEzOTU0MTYyNTk3NjU2LCJsZWZ0Ijo4LjMzMTc0NzA1NTA1MzcxMSwid2lkdGgiOjM4LjQ1NjgyMTQ0MTY1MDM5LCJoZWlnaHQiOjEuNTU4MzMyMjA0ODE4NzI1Nn19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjc5LjQ1ODYyNTc5MzQ1NzAzLCJsZWZ0Ijo4LjMzMTU5OTIzNTUzNDY2OCwid2lkdGgiOjQwLjUwOTY1ODgxMzQ3NjU2LCJoZWlnaHQiOjExLjU3MzY4NjU5OTczMTQ0NX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjkuNjU1MTM2MTA4Mzk4NDM4LCJsZWZ0Ijo1MS4yMDkyMTcwNzE1MzMyLCJ3aWR0aCI6NDAuNDg1NjQ1Mjk0MTg5NDUsImhlaWdodCI6MS4xODA1MTk1ODA4NDEwNjQ1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6MTEuMTM2OTkxNTAwODU0NDkyLCJsZWZ0Ijo1MS4yMDkyMTcwNzE1MzMyLCJ3aWR0aCI6NDAuNDgzMTk2MjU4NTQ0OTIsImhlaWdodCI6MS4xODA1MTk1ODA4NDEwNjQ1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6MTIuNjI1NTgzNjQ4NjgxNjQsImxlZnQiOjUxLjIwOTIxNzA3MTUzMzIsIndpZHRoIjo0MC40OTA3NTY5ODg1MjUzOSwiaGVpZ2h0IjoxLjE4MDUxOTU4MDg0MTA2NDV9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI2In19LHsib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlVtYXIiLCJmYW1pbHkiOiJEYXJheiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiLFoHRlZmFuIiwiZmFtaWx5IjoiQm9qbmVjIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllvdW5hcyIsImZhbWlseSI6IktoYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMDc3LTA0NzIiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzOTAvYWdyaWN1bHR1cmUxNTA1MDU1NCIsIm9wZW5hbGV4IjoiVzQ0MDgyMTk4MTEiLCJleGEiOiJodHRwczovL3d3dy5tZHBpLmNvbS8yMDc3LTA0NzIvMTUvNS81NTQifSwidGl0bGUiOiJFbmVyZ3ktRWZmaWNpZW50IFNtYXJ0IElycmlnYXRpb24gVGVjaG5vbG9naWVzOiBBIFBhdGh3YXkgdG8gV2F0ZXIgYW5kIEVuZXJneSBTdXN0YWluYWJpbGl0eSBpbiBBZ3JpY3VsdHVyZSIsImFic3RyYWN0IjoiVGhlIGFncmljdWx0dXJhbCBzZWN0b3IgZmFjZXMgY2hhbGxlbmdlcyBzdWNoIGFzIHdhdGVyIHNjYXJjaXR5LCBlbmVyZ3kgaW5lZmZpY2llbmN5LCBhbmQgZGVjbGluaW5nIHByb2R1Y3Rpdml0eSwgcGFydGljdWxhcmx5IGluIGFyaWQgcmVnaW9ucy4gVHJhZGl0aW9uYWwgaXJyaWdhdGlvbiBtZXRob2RzIGNvbnRyaWJ1dGUgdG8gcmVzb3VyY2UgZGVwbGV0aW9uIGFuZCBlbnZpcm9ubWVudGFsIGltcGFjdHMuIFNvbGFyLXBvd2VyZWQgc21hcnQgaXJyaWdhdGlvbiBzeXN0ZW1zIGludGVncmF0ZSBwcmVjaXNpb24gaXJyaWdhdGlvbiB3aXRoIHJlbmV3YWJsZSBlbmVyZ3ksIGltcHJvdmluZyB3YXRlciB1c2UgYW5kIHByb2R1Y3Rpdml0eS4gSW4gUGFraXN0YW4sIHdoZXJlIGFncmljdWx0dXJlIGNvbnRyaWJ1dGVzIDE5JSBvZiBncm9zcyBkb21lc3RpYyBwcm9kdWN0IGFuZCBlbXBsb3lzIDQwJSBvZiB0aGUgd29ya2ZvcmNlLCB0aGVzZSBjaGFsbGVuZ2VzIGFyZSBzZXZlcmUsIGVzcGVjaWFsbHkgaW4gd2F0ZXItc2NhcmNlIGFyZWFzIGxpa2UgdGhlIENob2xpc3RhbiBEZXNlcnQuIFRoaXMgc3R1ZHkgZXhhbWluZXMgdGhlIGltcGFjdCBvZiBzb2xhci1wb3dlcmVkIHNtYXJ0IGlycmlnYXRpb24gb24gYWdyaWN1bHR1cmFsIHByb2R1Y3Rpdml0eSwgd2F0ZXIgY29uc2VydmF0aW9uLCBhbmQgZW5lcmd5IGVmZmljaWVuY3kgaW4gdGhlIENob2xpc3RhbiBEZXNlcnQuIFVzaW5nIGEgcXVhbnRpdGF0aXZlIGNyb3NzLXNlY3Rpb25hbCBkZXNpZ24sIGRhdGEgd2VyZSBjb2xsZWN0ZWQgZnJvbSAzODQgZmFybWVycyB2aWEgc3RydWN0dXJlZCBxdWVzdGlvbm5haXJlcy4gU3RhdGlzdGljYWwgYW5hbHlzZXMsIGluY2x1ZGluZyBtdWx0aXBsZSBsaW5lYXIgcmVncmVzc2lvbiwgcGFpcmVkIHNhbXBsZSB0LXRlc3RzLCBhbmQgU3RydWN0dXJhbCBFcXVhdGlvbiBNb2RlbGluZyAoU0VNKSwgd2VyZSBjb25kdWN0ZWQuIEZpbmRpbmdzIHNob3cgc2lnbmlmaWNhbnQgaW1wcm92ZW1lbnRzIGluIGNyb3AgeWllbGQgKGZyb20gMy4wIHRvIDQuOCB0b25zL2hlY3RhcmUpIGFuZCByZWR1Y3Rpb25zIGluIHdhdGVyIGFuZCBlbmVyZ3kgY29uc3VtcHRpb24uIFJlZ3Jlc3Npb24gYW5hbHlzaXMgaGlnaGxpZ2h0ZWQgc3Ryb25nIHBvc2l0aXZlIGVmZmVjdHMgb24geWllbGQgYW5kIGVmZmljaWVuY3ksIHdoaWxlIFNFTSBjb25maXJtZWQgcmVkdWNlZCBlbnZpcm9ubWVudGFsIGltcGFjdCBhbmQgb3BlcmF0aW9uYWwgY29zdHMuIFRoZSBzdHVkeSBjb25jbHVkZXMgdGhhdCBzb2xhci1wb3dlcmVkIGlycmlnYXRpb24gZW5oYW5jZXMgcHJvZHVjdGl2aXR5LCBjb25zZXJ2ZXMgcmVzb3VyY2VzLCBhbmQgcHJvbW90ZXMgc3VzdGFpbmFiaWxpdHkuIFBvbGljeW1ha2VycyBzaG91bGQgcHJvdmlkZSBmaW5hbmNpYWwgaW5jZW50aXZlcywgaW52ZXN0IGluIHJlbmV3YWJsZSBpbmZyYXN0cnVjdHVyZSwgYW5kIGltcGxlbWVudCB0cmFpbmluZyBwcm9ncmFtcyB0byBzdXBwb3J0IGFkb3B0aW9uLiBDb2xsYWJvcmF0aXZlIGVmZm9ydHMgYXJlIGVzc2VudGlhbCBmb3Igc3VzdGFpbmFibGUgYWdyaWN1bHR1cmUgaW4gYXJpZCByZWdpb25zLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzkwL2FncmljdWx0dXJlMTUwNTA1NTQiLCJpc3N1ZWREYXRlIjoiMjAyNS0wMy0wNSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkFncmljdWx0dXJlIiwiaXNzdWUiOjUsInZvbHVtZSI6MTUsImltcGFjdEZhY3RvciI6IjUuMjYifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjo1NTQsImxhc3RQYWdlIjo1NTR9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiTXVsdGlkaXNjaXBsaW5hcnkgRGlnaXRhbCBQdWJsaXNoaW5nIEluc3RpdHV0ZSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NDgsImNpdGVkQnlDb3VudCI6Mn19LCJxdW90ZSI6IjEuIEludHJvZHVjdGlvbiBUaGUgYWdyaWN1bHR1cmFsIHNlY3RvciwgYSBjb3JuZXJzdG9uZSBvZiBnbG9iYWwgZm9vZCBzZWN1cml0eSBhbmQgZWNvbm9taWMgZGV2ZWxvcG1lbnQsIGZhY2VzIG1vdW50aW5nIGNoYWxsZW5nZXMgaW4gdGhlIDIxc3QgY2VudHVyeS4gQ2xpbWF0ZSBjaGFuZ2UsIHJhcGlkIHBvcHVsYXRpb24gZ3Jvd3RoLCBhbmQgZHdpbmRsaW5nIG5hdHVyYWwgcmVzb3VyY2VzIGhhdmUgaW50ZW5zaWZpZWQgdGhlIG5lZWQgZm9yIHN1c3RhaW5hYmxlIGFncmljdWx0dXJhbCBwcmFjdGljZXMuIEFtb25nIHRoZXNlIGNoYWxsZW5nZXMsIHdhdGVyIHNjYXJjaXR5IGFuZCBlbmVyZ3kgaW5lZmZpY2llbmN5IHN0YW5kIG91dCBhcyBjcml0aWNhbCBiYXJyaWVycyB0byBwcm9kdWN0aXZpdHkgYW5kIHN1c3RhaW5hYmlsaXR5LiBHbG9iYWxseSwgYWdyaWN1bHR1cmUgYWNjb3VudHMgZm9yIGFwcHJveGltYXRlbHkgNzAlIG9mIGZyZXNod2F0ZXIgd2l0aGRyYXdhbHMgYW5kIGlzIG9uZSBvZiB0aGUgbGFyZ2VzdCBjb25zdW1lcnMgb2YgZW5lcmd5IGluIG1hbnkgcmVnaW9ucyBbIDEsIDJdLiBUcmFkaXRpb25hbCBpcnJpZ2F0aW9uIHByYWN0aWNlcywgb2Z0ZW4gaW5lZmZpY2llbnQgYW5kIHJlc291cmNlLWludGVuc2l2ZSwgZXhhY2VyYmF0ZSB0aGVzZSBpc3N1ZXMsIGxlYWRpbmcgdG8gd2F0ZXIgd2FzdGFnZSwgZW5lcmd5IG92ZXJ1c2UsIGFuZCByZWR1Y2VkIGNyb3AgeWllbGRzLiJ9XQ%3D%3D. Infrastructure gaps, such as inadequate water conveyance networks and aging distribution systems, exacerbate inefficiencies, with traditional flood irrigation persisting due to the high upfront costs of retrofitting (Jumintono et al., 2024; Kushwaha et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjMwLCJzdWJ0eXBlIjp7fSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTWFuaXNoIiwiZmFtaWx5IjoiS3VzaHdhaGEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2hhbmthciIsImZhbWlseSI6IlNpbmdoIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlZpamF5IEthbnQiLCJmYW1pbHkiOiJTaW5naCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTaGFzaGFuayIsImZhbWlseSI6IkR3aXZlZGkifX1dLCJpZHMiOnsiaXNzbiI6WyIyNDU2LTE4NzgiXSwiaXNibiI6W10sImRvaSI6IjEwLjIyMTYxL2lqZWFiLjkyLjI3Iiwib3BlbmFsZXgiOiJXNDM5NjU2MzU4NCJ9LCJ0aXRsZSI6IlByZWNpc2lvbiBGYXJtaW5nOiBBIFJldmlldyBvZiBNZXRob2RzLCBUZWNobm9sb2dpZXMsIGFuZCBGdXR1cmUgUHJvc3BlY3RzIiwiYWJzdHJhY3QiOiJQcmVjaXNpb24gZmFybWluZywgb2Z0ZW4gcmVmZXJyZWQgdG8gYXMgcHJlY2lzaW9uIGFncmljdWx0dXJlLCBpcyBhIGZhcm1pbmcgbWFuYWdlbWVudCBjb25jZXB0IHRoYXQgc2Vla3MgdG8gbWF4aW1pemUgYWdyaWN1bHR1cmFsIG9wZXJhdGlvbnMgdmlhIHRoZSB1c2Ugb2YgdGVjaG5vbG9neSBpbiBvcmRlciB0byBhY2hpZXZlIGdyZWF0ZXIgbGV2ZWxzIG9mIGVmZmljaWVuY3ksIHByb2R1Y3Rpb24sIGFuZCBjb25zZXJ2YXRpb24gb2YgcmVzb3VyY2VzLiBUaGUgcHVycG9zZSBvZiB0aGlzIHN1cnZleSBzdHVkeSBpcyB0byBvZmZlciBhbiBvdmVydmlldyBvZiBwcmVjaXNpb24gZmFybWluZywgaW5jbHVkaW5nIHRvcGljcyBzdWNoIGFzIGl0cyBmdW5kYW1lbnRhbHMsIG1ldGhvZG9sb2dpZXMsIHRlY2hub2xvZ3ksIGFuZCBhcHBsaWNhdGlvbnMuIEl0IGludmVzdGlnYXRlcyB0aGUgcm9sZSB0aGF0IGEgdmFyaWV0eSBvZiB0ZWNobm9sb2dpZXMsIGluY2x1ZGluZyBnbG9iYWwgbmF2aWdhdGlvbiBzYXRlbGxpdGUgc3lzdGVtcyAoR05TUyksIGdlb2dyYXBoaWMgaW5mb3JtYXRpb24gc3lzdGVtcyAoR0lTKSwgcmVtb3RlIHNlbnNpbmcsIGFuZCBzZW5zb3IgdGVjaG5vbG9neSwgcGxheSBpbiBtYWtpbmcgaXQgcG9zc2libGUgdG8gaW1wbGVtZW50IHByZWNpc2lvbiBhZ3JpY3VsdHVyYWwgbWV0aG9kcy4gSW4gYWRkaXRpb24sIGl0IGFkZHJlc3NlcyB0aGUgYWR2YW50YWdlcywgZGlzYWR2YW50YWdlcywgYW5kIHBvdGVudGlhbCBmdXR1cmUgYXBwbGljYXRpb25zIG9mIHByZWNpc2lvbiBmYXJtaW5nIGluIHRoZSBjb250ZXh0IG9mIHNvbHZpbmcgaXNzdWVzIHJlbGF0ZWQgdG8gdGhlIHN1c3RhaW5hYmlsaXR5IG9mIGFncmljdWx0dXJlIGFuZCB0aGUgc2VjdXJpdHkgb2YgZm9vZCBzdXBwbGllcyBvbiBhIHdvcmxkd2lkZSBzY2FsZS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjIxNjEvaWplYWIuOTIuMjciLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vaWplYWIuY29tL3VwbG9hZF9kb2N1bWVudC9pc3N1ZV9maWxlcy8yN0lKRUFCLTEwNDIwMjQ4NC1QcmVjaXNpb25GYXJtaW5nLnBkZiIsImlzc3VlZERhdGUiOiIyMDI0LTAxLTAxIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiSW50ZXJuYXRpb25hbCBKb3VybmFsIG9mIEVudmlyb25tZW50IEFncmljdWx0dXJlIGFuZCBCaW90ZWNobm9sb2d5IiwiaXNzdWUiOjIsInZvbHVtZSI6OSwiaW1wYWN0RmFjdG9yIjoiMC42NSJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjI0MiwibGFzdFBhZ2UiOjI1M319LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJJbmZvZ2FpbiBQdWJsaWNhdGlvbiJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NCwiY2l0ZWRCeUNvdW50Ijo4fX0sInF1b3RlIjoiSUlJLiBCRU5FRklUUyBPRiBQUkVDSVNJT04gSVJSSUdBVElPTiAxLiBJbXByb3ZlZCBXYXRlciBVc2UgRWZmaWNpZW5jeTogUHJlY2lzaW9uIGlycmlnYXRpb24gb3B0aW1pemVzIHdhdGVyIHVzZSBlZmZpY2llbmN5IGJ5IGRlbGl2ZXJpbmcgd2F0ZXIgZGlyZWN0bHkgdG8gdGhlIHJvb3Qgem9uZSBvZiBjcm9wcywgbWluaW1pemluZyBsb3NzZXMgIGR1ZSB0byBldmFwb3JhdGlvbiwgcnVub2ZmLCBhbmQgZGVlcCBwZXJjb2xhdGlvbi4gQnkgIG1hdGNoaW5nIHdhdGVyIGFwcGxpY2F0aW9uIHJhdGVzIHRvIGNyb3Agd2F0ZXIgIHJlcXVpcmVtZW50cywgcHJlY2lzaW9uIGlycmlnYXRpb24gc3lzdGVtcyBlbnN1cmUgdGhhdCAgd2F0ZXIgaXMgdXNlZCBtb3JlIGVmZmljaWVudGx5LCBtYXhpbWl6aW5nIGNyb3AgeWllbGRzICBwZXIgdW5pdCBvZiB3YXRlciBhcHBsaWVkLiAyLiBFbmhhbmNlZCBDcm9wIFBlcmZvcm1hbmNlOiBCeSBwcm92aWRpbmcgY3JvcHMgd2l0aCBvcHRpbWFsIG1vaXN0dXJlIGxldmVscywgcHJlY2lzaW9uIGlycmlnYXRpb24gc3lzdGVtcyBwcm9tb3RlIGhlYWx0aHkgcm9vdCBkZXZlbG9wbWVudCwgYmFsYW5jZWQgZ3Jvd3RoLCAgYW5kIGltcHJvdmVkIGNyb3Agdmlnb3IuIENvbnNpc3RlbnQgc29pbCBtb2lzdHVyZSAgbGV2ZWxzIGhlbHAgcmVkdWNlIHdhdGVyIHN0cmVzcywgaW5jcmVhc2UgbnV0cmllbnQgIHVwdGFrZSwgYW5kIGVuaGFuY2UgY3JvcCByZXNpbGllbmNlIHRvIGVudmlyb25tZW50YWwgIHN0cmVzc2VzIHN1Y2ggYXMgZHJvdWdodCBhbmQgaGVhdC4gMy4gQ29zdCBTYXZpbmdzOiBQcmVjaXNpb24gaXJyaWdhdGlvbiBzeXN0ZW1zIGhlbHAgZmFybWVycyByZWR1Y2Ugd2F0ZXIsIGVuZXJneSwgYW5kIGxhYm9yIGNvc3RzIGFzc29jaWF0ZWQgd2l0aCBpcnJpZ2F0aW9uLiBCeSBvcHRpbWl6aW5nIHdhdGVyIHVzZSBlZmZpY2llbmN5IGFuZCAgbWluaW1pemluZyBpbnB1dCB3YXN0YWdlLCBwcmVjaXNpb24gaXJyaWdhdGlvbiBzeXN0ZW1zICBjYW4gbGVhZCB0byBzaWduaWZpY2FudCBjb3N0IHNhdmluZ3Mgb3ZlciB0aW1lLCAgaW1wcm92aW5nIHRoZSBwcm9maXRhYmlsaXR5IG9mIGFncmljdWx0dXJhbCBvcGVyYXRpb25zLiA0LiBFbnZpcm9ubWVudGFsIFN1c3RhaW5hYmlsaXR5OiAgUHJlY2lzaW9uICBpcnJpZ2F0aW9uIGNvbnRyaWJ1dGVzIHRvICBlbnZpcm9ubWVudGFsICBzdXN0YWluYWJpbGl0eSAgYnkgY29uc2VydmluZyB3YXRlciAgcmVzb3VyY2VzLCAgbWluaW1pemluZyAgd2F0ZXIgcG9sbHV0aW9uLCBhbmQgcmVkdWNpbmcgZ3JlZW5ob3VzZSBnYXMgZW1pc3Npb25zLiBCeSBhZG9wdGluZyBwcmVjaXNpb24gaXJyaWdhdGlvbiBwcmFjdGljZXMsIGZhcm1lcnMgY2FuICBtaW5pbWl6ZSB0aGVpciBlbnZpcm9ubWVudGFsIGZvb3RwcmludCBhbmQgY29udHJpYnV0ZSB0byB0aGUgIGxvbmctdGVybSAgc3VzdGFpbmFiaWxpdHkgIG9mICBhZ3JpY3VsdHVyYWwgcHJvZHVjdGlvbiBzeXN0ZW1zLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjQ0Ljg2NTY0MjU0NzYwNzQyLCJsZWZ0Ijo1My40MDgyNTI3MTYwNjQ0NSwid2lkdGgiOjM4LjI2MTc2ODM0MTA2NDQ1LCJoZWlnaHQiOjEuNjY5MjI3MzYxNjc5MDc3MX19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjQ2Ljg5NzkyMjUxNTg2OTE0LCJsZWZ0Ijo1Mi40MjA1NDM2NzA2NTQzLCJ3aWR0aCI6NDAuMjQwNzM3OTE1MDM5MDYsImhlaWdodCI6MS42MjU0NTQ3ODM0Mzk2MzYyfX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NDguNDkxNzkwNzcxNDg0Mzc1LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3Ljk4MDI3ODAxNTEzNjcyLCJoZWlnaHQiOjExLjAxMzcxMzgzNjY2OTkyMn19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjU5LjkzOTU0MDg2MzAzNzExLCJsZWZ0Ijo1Mi40MjA1NDM2NzA2NTQzLCJ3aWR0aCI6NDAuMjMwMjI0NjA5Mzc1LCJoZWlnaHQiOjEuNjI1NDU0NzgzNDM5NjM2Mn19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjYxLjUzMzQxNjc0ODA0Njg3NSwibGVmdCI6NTQuNjk4MzEwODUyMDUwNzgsIndpZHRoIjozNy45NjMxMjMzMjE1MzMyLCJoZWlnaHQiOjkuNDM1MTcwMTczNjQ1MDJ9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo3MS40MTY4Nzc3NDY1ODIwMywibGVmdCI6NTIuNDIwNTQzNjcwNjU0Mywid2lkdGgiOjQwLjIxMzMxMDI0MTY5OTIyLCJoZWlnaHQiOjEuNjI1NDU4MzU5NzE4MzIyOH19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjczLjAxMDc1NzQ0NjI4OTA2LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3Ljk0NTc4NTUyMjQ2MDk0LCJoZWlnaHQiOjcuODYzNzY0Mjg2MDQxMjZ9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo4MS4zMDg1NTU2MDMwMjczNCwibGVmdCI6NTIuNDIwNTQzNjcwNjU0Mywid2lkdGgiOjQwLjI0MDM3NTUxODc5ODgzLCJoZWlnaHQiOjEuNjI1NDU4MzU5NzE4MzIyOH19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjgyLjkxOTA1OTc1MzQxNzk3LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3LjkzNjQxNjYyNTk3NjU2LCJoZWlnaHQiOjEuNTc4MTMzNTgzMDY4ODQ3N319LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjg0LjQ4NjkwMDMyOTU4OTg0LCJsZWZ0Ijo1NC42OTgzMTA4NTIwNTA3OCwid2lkdGgiOjM3LjkzNDc1NzIzMjY2NjAxNiwiaGVpZ2h0IjoxLjU3ODEzMzU4MzA2ODg0Nzd9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo4Ni4wNTQ3NDg1MzUxNTYyNSwibGVmdCI6NTQuNjk4MzEwODUyMDUwNzgsIndpZHRoIjozNy45Nzg0MzE3MDE2NjAxNTYsImhlaWdodCI6NC43MTM4MTkwMjY5NDcwMjE1fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6OC40MjAyOTI4NTQzMDkwODIsImxlZnQiOjEwLjAyMjE3MjkyNzg1NjQ0NSwid2lkdGgiOjM3Ljk0NjgzMDc0OTUxMTcyLCJoZWlnaHQiOjEuNTc4MTM1MjUxOTk4OTAxNH19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjkuOTg4MTMyNDc2ODA2NjQsImxlZnQiOjEwLjAyMjE3MjkyNzg1NjQ0NSwid2lkdGgiOjEzLjg2MjQ2MTA5MDA4Nzg5LCJoZWlnaHQiOjEuNTc4MTM1MjUxOTk4OTAxNH19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjI0OSJ9fSx7Im9yaWdpbiI6NCwid29yayI6eyJ0eXBlIjo4LCJzdWJ0eXBlIjp7Im9uZW9mS2luZCI6ImFydGljbGVUeXBlIiwiYXJ0aWNsZVR5cGUiOjF9LCJjb250cmlidXRvcnMiOlt7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJKdW1pbnRvbm8iLCJmYW1pbHkiOiJKdW1pbnRvbm8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTnVybmF3YXR5IiwiZmFtaWx5IjoiTnVybmF3YXR5In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik51Z3JvaG8gVHJpc251IiwiZmFtaWx5IjoiQnJhdGEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiRGluaSIsImZhbWlseSI6IlJhaG1pYXRpIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhhZGkiLCJmYW1pbHkiOiJTdXNpbG8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQS4gQS4iLCJmYW1pbHkiOiJIdXNzZW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIxNDI5LTc0MjYiLCIyMDgzLTQ1MzUiXSwiaXNibiI6W10sImRvaSI6IjEwLjI0NDI1L2p3bGQuMjAyNC4xNTE3OTgiLCJvcGVuYWxleCI6Ilc0NDA1NDc3MTAwIn0sInRpdGxlIjoiQ29tcGFyYXRpdmUgYW5hbHlzaXMgb2YgcHJlY2lzaW9uLCBkZWZpY2l0LCBhbmQgZHJpcCBpcnJpZ2F0aW9uIGZvciB3YXRlciBlZmZpY2llbmN5IGluIElyYXEiLCJhYnN0cmFjdCI6IlN1c3RhaW5hYmxlIGlycmlnYXRpb24gcHJhY3RpY2VzIGFyZSBjcml0aWNhbCBmb3IgYWRkcmVzc2luZyB0aGUgY2hhbGxlbmdlcyBvZiB3YXRlciBzY2FyY2l0eSBhbmQgZm9vZCBpbnNlY3VyaXR5IGluIHRoZSBmYWNlIG9mIGNsaW1hdGUgY2hhbmdlIGFuZCBncm93aW5nIGdsb2JhbCBmb29kIGRlbWFuZC4gVGhpcyBzdHVkeSBpbnZlc3RpZ2F0ZWQgdGhlIHBvdGVudGlhbCBvZiBwcmVjaXNpb24gaXJyaWdhdGlvbiAoUEkpLCBkZWZpY2l0IGlycmlnYXRpb24gKERJKSwgYW5kIGRyaXAgaXJyaWdhdGlvbiAoRFJJKSB0ZWNobmlxdWVzIHRvIGVuaGFuY2UgY3JvcCBwcm9kdWN0aXZpdHkgYW5kIHdhdGVyIHVzZSBlZmZpY2llbmN5IChXVUUpIGNvbXBhcmVkIHRvIGNvbnZlbnRpb25hbCBmbG9vZCBpcnJpZ2F0aW9uIChDRkkpIGluIHRocmVlIGRpc3RpbmN0IGFncm9lY29sb2dpY2FsIHpvbmVzIG9mIElyYXEuIEZpZWxkIGV4cGVyaW1lbnRzIHdlcmUgY29uZHVjdGVkIHVzaW5nIGEgcmFuZG9taXNlZCBjb21wbGV0ZSBibG9jayBkZXNpZ24gd2l0aCB3aGVhdCwgbWFpemUsIGFuZCByaWNlIGNyb3BzLiBUaGUgcmVzdWx0cyBkZW1vbnN0cmF0ZWQgdGhhdCBQSSBzaWduaWZpY2FudGx5IGluY3JlYXNlZCBjcm9wIGdyb3d0aCBwYXJhbWV0ZXJzLCBncmFpbiB5aWVsZCwgYW5kIFdVRSBhY3Jvc3MgYWxsIHpvbmVzLCB3aXRoIHlpZWxkIGltcHJvdmVtZW50cyBvZiAzM%2BKAkzM4JSBhbmQgV1VFIGluY3JlYXNlcyBvZiA0NuKAkzUxJSBpbiBjb250cmFzdCB3aXRoIENGSS4gVGhlIERJIGFuZCBEUkkgdHJlYXRtZW50cyBhbHNvIG91dHBlcmZvcm1lZCBDRkksIGFsYmVpdCB0byBhIGxlc3NlciBleHRlbnQuIFJlbW90ZSBzZW5zaW5nLWRlcml2ZWQgdmVnZXRhdGlvbiBpbmRpY2VzIHN0cm9uZ2x5IGNvcnJlbGF0ZWQgd2l0aCBjcm9wIGdyb3d0aCBwYXJhbWV0ZXJzIGFuZCB5aWVsZCwgd2hpbGUgaHlkcm9sb2dpY2FsIG1vZGVsbGluZyByZXZlYWxlZCByZWR1Y2VkIGV2YXBvdHJhbnNwaXJhdGlvbiBhbmQgc3VyZmFjZSBydW5vZmYgdW5kZXIgdGhlIFBJIHRyZWF0bWVudC4gVGhlIHN1c3RhaW5hYmxlIGlycmlnYXRpb24gcHJhY3RpY2VzIHJlc3VsdGVkIGluIHN1YnN0YW50aWFsIHdhdGVyIHNhdmluZ3Mgb2YgMjDigJMzMCUgY29tcGFyZWQgdG8gQ0ZJLiBUaGVzZSBmaW5kaW5ncyBoaWdobGlnaHQgdGhlIGltcG9ydGFuY2Ugb2YgYWRvcHRpbmcgZWZmaWNpZW50IGlycmlnYXRpb24gdGVjaG5pcXVlcywgYWxvbmcgd2l0aCBhIGhvbGlzdGljIGFwcHJvYWNoIGVuY29tcGFzc2luZyB0ZWNobm9sb2dpY2FsIGlubm92YXRpb25zLCBjYXBhY2l0eSBidWlsZGluZywgYW5kIHN0YWtlaG9sZGVyIGVuZ2FnZW1lbnQsIHRvIHByb21vdGUgd2F0ZXItIGVmZmljaWVudCBhZ3JpY3VsdHVyZSBhbmQgZW5zdXJlIGZvb2Qgc2VjdXJpdHkgaW4gd2F0ZXItc2NhcmNlIHJlZ2lvbnMuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjI0NDI1L2p3bGQuMjAyNC4xNTE3OTgiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4yNDQyNS9qd2xkLjIwMjQuMTUxNzk4IiwiaXNzdWVkRGF0ZSI6IjIwMjQtMTItMTQiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTEwIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJKb3VybmFsIG9mIFdhdGVyIGFuZCBMYW5kIERldmVsb3BtZW50IiwiaW1wYWN0RmFjdG9yIjoiMS40NiJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjEzMCwibGFzdFBhZ2UiOjEzNn19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJEZSBHcnV5dGVyIE9wZW4ifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjMyLCJjaXRlZEJ5Q291bnQiOjF9fSwicXVvdGUiOiJJbiB0aGlzIGNvbnRleHQsIHRoZSBhZG9wdGlvbiBvZiAgc3VzdGFpbmFibGUgaXJyaWdhdGlvbiBwcmFjdGljZXMgaGFzIGVtZXJnZWQgYXMgYSBjcml0aWNhbCBzdHJhdGVneSAgZm9yIGVuaGFuY2luZyBmb29kIHNlY3VyaXR5IHdoaWxlIHByb21vdGluZyB3YXRlciBjb25zZXJ2YXRpb24gIGluIGFncmljdWx0dXJhbCBsYW5kc2NhcGVzLiAgSXJyaWdhdGlvbiBwbGF5cyBhIHZpdGFsIHJvbGUgaW4gZ2xvYmFsIGZvb2QgcHJvZHVjdGlvbiwgd2l0aCAgaXJyaWdhdGVkIGFncmljdWx0dXJlIGFjY291bnRpbmcgZm9yIGFwcHJveGltYXRlbHkgNDAlIG9mIHRoZSAgZm9vZCBzdXBwbHkgKFdyYWNoaWVuIGRlLCBTY2h1bHR6IGFuZCBHb2xpLCAyMDIxOyBTY2htaXR0LCBSb3NhICBhbmQgRGFpbHksIDIwMjIpLiBIb3dldmVyLCB0cmFkaXRpb25hbCBpcnJpZ2F0aW9uIG1ldGhvZHMsIHN1Y2ggYXMgIGZsb29kIGFuZCBmdXJyb3cgaXJyaWdhdGlvbiwgYXJlIG9mdGVuIGluZWZmaWNpZW50LCBsZWFkaW5nIHRvICBzdWJzdGFudGlhbCB3YXRlciBsb3NzZXMgdGhyb3VnaCBldmFwb3JhdGlvbiwgcnVub2ZmLCBhbmQgZGVlcCAgcGVyY29sYXRpb24gKEFtYm9tc2EsIDIwMjApLiBNb3Jlb3ZlciwgdGhlIGV4Y2Vzc2l2ZSBhcHBsaWNhLXRpb24gb2Ygd2F0ZXIgY2FuIHJlc3VsdCBpbiBzb2lsIHNhbGluaXNhdGlvbiwgbnV0cmllbnQgbGVhY2hpbmcsIGFuZCAgZ3JvdW5kd2F0ZXIgZGVwbGV0aW9uLCBmdXJ0aGVyIGNvbXByb21pc2luZyB0aGUgbG9uZy10ZXJtICBzdXN0YWluYWJpbGl0eSBvZiBhZ3JpY3VsdHVyYWwgc3lzdGVtcyAoWWluIGV0IGFsLiwgMjAyMikuIFRvIGFkZHJlc3MgIHRoZXNlIGNoYWxsZW5nZXMsIHJlc2VhcmNoZXJzIGFuZCBwcmFjdGl0aW9uZXJzIGhhdmUgYmVlbiBleHBsb3ItaW5nIGlubm92YXRpdmUgaXJyaWdhdGlvbiB0ZWNobmlxdWVzIGFuZCB3YXRlciBtYW5hZ2VtZW50ICBzdHJhdGVnaWVzIHRoYXQgY2FuIG9wdGltaXNlIHdhdGVyIHVzZSBlZmZpY2llbmN5IChXVUUpIHdoaWxlICBtYWludGFpbmluZyBvciBlbmhhbmNpbmcgY3JvcCBwcm9kdWN0aXZpdHkuICBQcmVjaXNpb24gaXJyaWdhdGlvbiAoUEkpIG9mZmVycyBhIHByb21pc2luZyBzb2x1dGlvbiwgIGludm9sdmluZyB0aGUgdGFyZ2V0ZWQgYXBwbGljYXRpb24gb2Ygd2F0ZXIgc3BlY2lmaWNhbGx5IHRvIHRoZSAgbmVlZHMgb2YgaW5kaXZpZHVhbCBwbGFudHMgb3IgY3JvcCB6b25lcyAoWmlua2VybmFnZWwgZXQgYWwuLCAyMDIwKS4gICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MjIuODg2NzM1OTE2MTM3Njk1LCJsZWZ0Ijo5LjA0NzYwMTY5OTgyOTEwMiwid2lkdGgiOjQwLjM0NjIyMTkyMzgyODEyNSwiaGVpZ2h0IjoxLjQ1MDU0MTAxOTQzOTY5NzN9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyNC4zNTExMjM4MDk4MTQ0NTMsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQ1NzUyNzE2MDY0NDUsImhlaWdodCI6MS4zNjAwMTQ3OTYyNTcwMTl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyNS43MjQ5ODcwMzAwMjkyOTcsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQ1NzU2NTMwNzYxNzIsImhlaWdodCI6MS4zNjAwMTQ3OTYyNTcwMTl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyNy4wOTg4NTAyNTAyNDQxNCwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjoxNi40MTgxNjEzOTIyMTE5MTQsImhlaWdodCI6MS4zNjAwMTQ3OTYyNTcwMTl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyOC40NzI3MDk2NTU3NjE3MiwibGVmdCI6MTIuMzgwODgwMzU1ODM0OTYxLCJ3aWR0aCI6MzcuMDEyMTg0MTQzMDY2NDA2LCJoZWlnaHQiOjEuMzYwMDE0Nzk2MjU3MDE5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MjkuODQ2NTcyODc1OTc2NTYyLCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjM0MDY0ODY1MTEyMzA1LCJoZWlnaHQiOjEuMzYwMDE2NzAzNjA1NjUxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMxLjIxNDAzNjk0MTUyODMyLCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjM0NDg1NjI2MjIwNzAzLCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMyLjU4NzkwMjA2OTA5MTgsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQ2MDU3ODkxODQ1NywiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozMy45NjE3NjE0NzQ2MDkzNzUsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQwNDg0NjE5MTQwNjI1LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjM1LjMzNTYyNDY5NDgyNDIyLCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjMzOTc0NDU2Nzg3MTA5NCwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNi43MDk0ODc5MTUwMzkwNiwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4wMDE1OTA3Mjg3NTk3NjYsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzguMDgzMzQ3MzIwNTU2NjQsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQ1NzU2NTMwNzYxNzIsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzkuNDU3MjEwNTQwNzcxNDg0LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjM0MzA0ODA5NTcwMzEyNSwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0MC43NDA1NDcxODAxNzU3OCwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU0NTUxNjk2Nzc3MzQsImhlaWdodCI6MS40NTA1NDI4MDc1NzkwNDA1fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDIuMjA0OTMzMTY2NTAzOTA2LCJsZWZ0Ijo5LjA0NzYwMTY5OTgyOTEwMiwid2lkdGgiOjQwLjAwMzI1MDEyMjA3MDMxLCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQzLjU3ODc5NjM4NjcxODc1LCJsZWZ0Ijo5LjA0NzYwMTY5OTgyOTEwMiwid2lkdGgiOjQwLjMzNzkyMTE0MjU3ODEyNSwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0NC44NjIxMzMwMjYxMjMwNSwibGVmdCI6OS4wNDc2MDE2OTk4MjkxMDIsIndpZHRoIjo0MC4zNDUzMTQwMjU4Nzg5MDYsImhlaWdodCI6MS40NTA1NDI4MDc1NzkwNDA1fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDYuMzI2NTE5MDEyNDUxMTcsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6MjcuOTY3NDQ3MjgwODgzNzksImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDcuNzAwMzgyMjMyNjY2MDE2LCJsZWZ0IjoxMi4zODA4Nzg0NDg0ODYzMjgsIndpZHRoIjozNy4wMzQ3ODYyMjQzNjUyMzQsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDkuMDc0MjQ1NDUyODgwODYsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzQwNjQ0ODM2NDI1NzgsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTAuMzU3NTgyMDkyMjg1MTU2LCJsZWZ0Ijo5LjA0NzYwMTY5OTgyOTEwMiwid2lkdGgiOjQwLjM0NDg1MjQ0NzUwOTc2NiwiaGVpZ2h0IjoxLjQ1MDU0MjgwNzU3OTA0MDV9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIxMzEifX1d. Overcoming these requires scalable, low-power solutions like solar-powered drip systems and LoRaWAN networks to enable precision in water-scarce areas (Xie, 2025)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjMwLCJzdWJ0eXBlIjp7fSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQ2hlbiIsImZhbWlseSI6IlhpZSJ9fV0sImlkcyI6eyJpc3NuIjpbIjE2NjQtNDYyWCJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMzM4OS9mcGxzLjIwMjUuMTY3NTY1NyIsInBtaWQiOiI0MTAzNjM5MyIsIm9wZW5hbGV4IjoiVzQ0MTQyNDgzMDcifSwidGl0bGUiOiJUaGUgcm9sZSBvZiBtb2Rlcm4gYWdyaWN1bHR1cmFsIHRlY2hub2xvZ2llcyBpbiBpbXByb3ZpbmcgYWdyaWN1bHR1cmFsIHByb2R1Y3Rpdml0eSBhbmQgbGFuZCB1c2UgZWZmaWNpZW5jeSIsImFic3RyYWN0IjoiTW9kZXJuIGFncmljdWx0dXJhbCB0ZWNobm9sb2dpZXMgYXJlIGNydWNpYWwgZm9yIGFkZHJlc3NpbmcgZ2xvYmFsIGZvb2Qgc2VjdXJpdHkgYW5kIGVudmlyb25tZW50YWwgc3VzdGFpbmFiaWxpdHkgY2hhbGxlbmdlcyBhbWlkc3QgYSBncm93aW5nIHBvcHVsYXRpb24gYW5kIGNsaW1hdGUgY2hhbmdlLiBUaGVzZSBpbm5vdmF0aW9ucywgaW5jbHVkaW5nIHByZWNpc2lvbiBhZ3JpY3VsdHVyZSwgYmlvdGVjaG5vbG9neSwgc21hcnQgaXJyaWdhdGlvbiwgYXV0b21hdGlvbiwgdmVydGljYWwgZmFybWluZywgYW5kIGFydGlmaWNpYWwgaW50ZWxsaWdlbmNlIChBSSksIHNpZ25pZmljYW50bHkgZW5oYW5jZSBwcm9kdWN0aXZpdHkgYW5kIGxhbmQgdXNlIGVmZmljaWVuY3kuIFByZWNpc2lvbiBhZ3JpY3VsdHVyZSwgdXRpbGl6aW5nIEdQUywgZHJvbmVzLCBhbmQgSW9ULCBpbXByb3ZlcyB5aWVsZHMgYnkgMjDigJMzMCUgYW5kIGN1dHMgaW5wdXQgd2FzdGUgYnkgNDDigJM2MCUuIEJpb3RlY2hub2xvZ3ksIHdpdGggQ1JJU1BSIGFuZCBHTU9zLCBkZWxpdmVycyBkcm91Z2h0IGFuZCBwZXN0LXJlc2lzdGFudCBjcm9wcywgc3RhYmlsaXppbmcgeWllbGRzLCBhcyBzZWVuIHdpdGggQnQgY290dG9uIHJlZHVjaW5nIHBlc3RpY2lkZSB1c2UgYnkgNTAlIGluIEluZGlhLiBTbWFydCBpcnJpZ2F0aW9uIGJvb3N0cyB3YXRlciBlZmZpY2llbmN5IGJ5IDQw4oCTNjAlLCB3aGlsZSBhdXRvbWF0aW9uIGFuZCByb2JvdGljcyBtaXRpZ2F0ZSBsYWJvciBzaG9ydGFnZXMgYW5kIHJlZHVjZSBjb3N0cyBieSAyNSUuIFZlcnRpY2FsIGZhcm1pbmcgaW5jcmVhc2VzIHlpZWxkcyAxMOKAkzIwIHRpbWVzIHdpdGggOTUlIGxlc3MgbGFuZCBhbmQgd2F0ZXIsIHN1cHBvcnRpbmcgdXJiYW4gZm9vZCBzZWN1cml0eS4gQUkgYW5hbHl0aWNzIGVuaGFuY2UgZGVjaXNpb24tbWFraW5nIHdpdGggb3ZlciA5MCUgYWNjdXJhY3kgaW4gZm9yZWNhc3RpbmcgYW5kIHJlc291cmNlIGFsbG9jYXRpb24uIERlc3BpdGUgdGhlc2UgYmVuZWZpdHMsIGhpZ2ggY29zdHMsIHRlY2hub2xvZ2ljYWwgaWxsaXRlcmFjeSwgYW5kIHJlZ3VsYXRvcnkgaXNzdWVzIGhpbmRlciBhZG9wdGlvbiwgZXNwZWNpYWxseSBhbW9uZyBzbWFsbGhvbGRlcnMuIFBvbGljeSBzdXBwb3J0LCBwdWJsaWMtcHJpdmF0ZSBwYXJ0bmVyc2hpcHMsIGFuZCB0cmFpbmluZyBhcmUgdml0YWwgZm9yIGJyb2FkZXIgdGVjaG5vbG9neSBhY2Nlc3MgYW5kIGZhaXIgYmVuZWZpdHMuIEludGVncmF0aW5nIHJlbmV3YWJsZSBlbmVyZ3kgYW5kIGNpcmN1bGFyIGVjb25vbXkgcHJpbmNpcGxlcyBpbnRvIGFnZ3JvdGVjaCBwcmVzZW50cyBhIHBhdGggdG8gc3VzdGFpbmFiaWxpdHkuIFRoaXMgcmV2aWV3IGhpZ2hsaWdodHMgdGhlIHRyYW5zZm9ybWF0aXZlIHBvdGVudGlhbCBvZiBtb2Rlcm4gdGVjaG5vbG9naWVzIGZvciBzdXN0YWluYWJsZSBpbnRlbnNpZmljYXRpb24sIGluY3JlYXNpbmcgcHJvZHVjdGl2aXR5IHdpdGhvdXQgZXhwYW5kaW5nIGZhcm1sYW5kLCB3aGlsZSBsZXNzZW5pbmcgZW52aXJvbm1lbnRhbCBpbXBhY3RzLiBJdCB1bmRlcnNjb3JlcyB0aGUgbmVlZCBmb3IgY29vcmRpbmF0ZWQgZWZmb3J0cyB0byBvdmVyY29tZSBhZG9wdGlvbiBjaGFsbGVuZ2VzIGFuZCBoYXJuZXNzIHRoZXNlIGlubm92YXRpb25zIGZvciBnbG9iYWwgZm9vZCBzZWN1cml0eSBhbmQgY2xpbWF0ZSByZXNpbGllbmNlLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzg5L2ZwbHMuMjAyNS4xNjc1NjU3IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5mcm9udGllcnNpbi5vcmcvam91cm5hbHMvcGxhbnQtc2NpZW5jZS9hcnRpY2xlcy8xMC4zMzg5L2ZwbHMuMjAyNS4xNjc1NjU3L3BkZiIsImlzc3VlZERhdGUiOiIyMDI1LTA5LTE2IiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiRnJvbnRpZXJzIGluIFBsYW50IFNjaWVuY2UiLCJ2b2x1bWUiOjE2LCJpbXBhY3RGYWN0b3IiOiI3LjAwIn0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkZyb250aWVycyBNZWRpYSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6ODgsImNpdGVkQnlDb3VudCI6MH19LCJxdW90ZSI6IlN5bnRoZXRpYyBiaW9sb2d5IGlzIGFsc28gY29udHJpYnV0aW5nLCB3aXRoIG1pY3JvYmlhbCBjb25zb3J0aWEgZW5naW5lZXJlZCB0byBlbmhhbmNlIHNvaWwgaGVhbHRoIGFuZCBjcm9wIHJlc2lsaWVuY2UgKExhd3NvbiBldCBhbC4sIDIwMTkpLiBEZXNwaXRlIHRoZXNlIGFkdmFuY2VzLCBlcXVpdGFibGUgYWNjZXNzIHJlbWFpbnMgYSBjaGFsbGVuZ2UsIGFzIHNtYWxsaG9sZGVyIGZhcm1lcnMgaW4gZGV2ZWxvcGluZyByZWdpb25zIG9mdGVuIGxhY2sgYWNjZXNzIHRvIHBhdGVudGVkIHRlY2hub2xvZ2llcyAoVG9lbm5pZXNzZW4gZXQgYWwuLCAyMDA4KS4gSW5pdGlhdGl2ZXMgbGlrZSB0aGUgT3BlbkNSSVNQUiBwcm9qZWN0IGFpbSB0byBkZW1vY3JhdGl6ZSBnZW5lLWVkaXRpbmcgdG9vbHMgKENvaGVuLCAyMDIzKSwgd2hpbGUgcHVibGljLXNlY3RvciBwYXJ0bmVyc2hpcHMgKGUuZy4sIENJTU1ZVOKAmXMgZHJvdWdodC10b2xlcmFudCBtYWl6ZSkgZW5zdXJlIGFmZm9yZGFiaWxpdHkuIEFzIGNsaW1hdGUgY2hhbmdlIGludGVuc2lmaWVzLCB0aGVzZSBiaW90ZWNobm9sb2dpZXMgd2lsbCBiZSBpbmRpc3BlbnNhYmxlIGZvciBhY2hpZXZpbmcgc3VzdGFpbmFibGUgaW50ZW5zaWZpY2F0aW9uLCBidXQgdGhlaXIgc3VjY2VzcyBoaW5nZXMgb24gZ2xvYmFsIGNvbGxhYm9yYXRpb24sIGV0aGljYWwgZ292ZXJuYW5jZSwgYW5kIGluY2x1c2l2ZSBpbm5vdmF0aW9uLiAzLjMgU21hcnQgaXJyaWdhdGlvbiAmIHdhdGVyIG1hbmFnZW1lbnQgV2F0ZXIgc2NhcmNpdHkgcG9zZXMgb25lIG9mIHRoZSBtb3N0IHNpZ25pZmljYW50IGNoYWxsZW5nZXMgdG8gZ2xvYmFsIGFncmljdWx0dXJlLCB3aXRoIDcwJSBvZiBmcmVzaHdhdGVyIHdpdGhkcmF3YWxzIGFsbG9jYXRlZCB0byBpcnJpZ2F0aW9uLCBvZnRlbiBpbmVmZmljaWVudGx5IChGQU8sIDIwMjApLiBTbWFydCBpcnJpZ2F0aW9uIHN5c3RlbXMsIGludGVncmF0aW5nIElvVC1iYXNlZCBzZW5zb3JzIGFuZCBkcmlwIGlycmlnYXRpb24gdGVjaG5vbG9naWVzLCBhcmUgdHJhbnNmb3JtaW5nIHdhdGVyIG1hbmFnZW1lbnQgYnkgb3B0aW1pemluZyB1c2FnZSwgcmVkdWNpbmcgd2FzdGUsIGFuZCBlbmhhbmNpbmcgY3JvcCB5aWVsZHMuIFNvaWwgbW9pc3R1cmUgc2Vuc29ycywgc3VjaCBhcyBjYXBhY2l0aXZlIHByb2JlcyBhbmQgdGVuc2lvbWV0ZXJzLCBwcm92aWRlIHJlYWwtdGltZSBkYXRhIG9uIHdhdGVyIGF2YWlsYWJpbGl0eSBhdCB2YXJ5aW5nIHJvb3QgZGVwdGhzLCBlbmFibGluZyBwcmVjaXNlIGlycmlnYXRpb24gc2NoZWR1bGluZy4gVGhlc2Ugc2Vuc29ycyB0cmFuc21pdCBkYXRhIHZpYSBMb1JhV0FOIG9yIGNlbGx1bGFyIG5ldHdvcmtzIHRvIGNsb3VkIHBsYXRmb3Jtcywgd2hlcmUgQUkgYWxnb3JpdGhtcyBhbmFseXplIHRyZW5kcyBhbmQgYXV0b21hdGUgaXJyaWdhdGlvbiBkZWNpc2lvbnMgKEdvYXAgZXQgYWwuLCAyMDE4KS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo1Ny4xNzU2NDM5MjA4OTg0NCwibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ5MzE3NTUwNjU5MTgsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NTguNjY0MjM0MTYxMzc2OTUsImxlZnQiOjguMzMxNDQyODMyOTQ2Nzc3LCJ3aWR0aCI6NDAuNDkzOTUzNzA0ODMzOTg0LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjYwLjE0NjA4MDAxNzA4OTg0NCwibGVmdCI6OC4zMzE0NDE4NzkyNzI0NjEsIndpZHRoIjo0MC40OTc1ODE0ODE5MzM1OTQsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NjEuNjM0Njc3ODg2OTYyODksImxlZnQiOjguMzMxNDQxODc5MjcyNDYxLCJ3aWR0aCI6NDAuNDk1OTA2ODI5ODMzOTg0LCJoZWlnaHQiOjEuMTgwNTE2OTU4MjM2Njk0M319LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjYzLjExNjUyNzU1NzM3MzA1LCJsZWZ0Ijo4LjMzMTQ0MjgzMjk0Njc3Nywid2lkdGgiOjQwLjQ5NzU3NzY2NzIzNjMzLCJoZWlnaHQiOjEuMTgwNTE2OTU4MjM2Njk0M319LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjY0LjYwNTExNzc5Nzg1MTU2LCJsZWZ0Ijo4LjMzMTQzOTk3MTkyMzgyOCwid2lkdGgiOjQwLjQ4ODM0NjA5OTg1MzUxNiwiaGVpZ2h0IjoxLjE4MDUxNjk1ODIzNjY5NDN9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo2Ni4wODY5Njc0NjgyNjE3MiwibGVmdCI6OC4zMzE0NDE4NzkyNzI0NjEsIndpZHRoIjo0MC40ODMyMDc3MDI2MzY3MiwiaGVpZ2h0IjoxLjE4MDUxNjk1ODIzNjY5NDN9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo2Ny41NzU1NTM4OTQwNDI5NywibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ4MzA1MTMwMDA0ODgzLCJoZWlnaHQiOjEuMTgwNTE2OTU4MjM2Njk0M319LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjY5LjA1NzQwMzU2NDQ1MzEyLCJsZWZ0Ijo4LjMzMTU5MTYwNjE0MDEzNywid2lkdGgiOjQwLjQ4MTg2MTExNDUwMTk1LCJoZWlnaHQiOjEuMTgwNTE2OTU4MjM2Njk0M319LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjcwLjU0NTk5NzYxOTYyODksImxlZnQiOjguMzMxNTg5Njk4NzkxNTA0LCJ3aWR0aCI6NDAuNDg2MDgzOTg0Mzc1LCJoZWlnaHQiOjEuMTgwNTE2OTU4MjM2Njk0M319LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjcyLjAyNzg0NzI5MDAzOTA2LCJsZWZ0Ijo4LjMzMTQzOTAxODI0OTUxMiwid2lkdGgiOjM4LjIxMTg3MjEwMDgzMDA4LCJoZWlnaHQiOjEuMTgwNTE2OTU4MjM2Njk0M319LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjc2LjEzOTU0MTYyNTk3NjU2LCJsZWZ0Ijo4LjMzMTc0NzA1NTA1MzcxMSwid2lkdGgiOjM4LjQ1NjgyMTQ0MTY1MDM5LCJoZWlnaHQiOjEuNTU4MzMyMjA0ODE4NzI1Nn19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjc5LjQ1ODYyNTc5MzQ1NzAzLCJsZWZ0Ijo4LjMzMTU5OTIzNTUzNDY2OCwid2lkdGgiOjQwLjUwOTY1ODgxMzQ3NjU2LCJoZWlnaHQiOjExLjU3MzY4NjU5OTczMTQ0NX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjkuNjU1MTM2MTA4Mzk4NDM4LCJsZWZ0Ijo1MS4yMDkyMTcwNzE1MzMyLCJ3aWR0aCI6NDAuNDg1NjQ1Mjk0MTg5NDUsImhlaWdodCI6MS4xODA1MTk1ODA4NDEwNjQ1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6MTEuMTM2OTkxNTAwODU0NDkyLCJsZWZ0Ijo1MS4yMDkyMTcwNzE1MzMyLCJ3aWR0aCI6NDAuNDgzMTk2MjU4NTQ0OTIsImhlaWdodCI6MS4xODA1MTk1ODA4NDEwNjQ1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6MTIuNjI1NTgzNjQ4NjgxNjQsImxlZnQiOjUxLjIwOTIxNzA3MTUzMzIsIndpZHRoIjo0MC40OTA3NTY5ODg1MjUzOSwiaGVpZ2h0IjoxLjE4MDUxOTU4MDg0MTA2NDV9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI2In19XQ%3D%3D.

### Table 11: Real-Time Monitoring Parameters

| Parameter | Sensor Type | Management Action |
| --- | --- | --- |
| Soil Moisture | Tensiometers/IoT sensors | Trigger or stop irrigation |
| Crop Health | Satellite/Infrared | Adjust nutrient/water delivery |
| Evaporation Rate | Weather station data | Scheduling to avoid midday peaks |
| Humidity | Atmospheric sensors | Forecast disease risks |
| Temperature | Field-level sensors | Protect against frost/heat |
| Nutrient Level | Soil chemical sensors | Precise fertilization |
| Flow Rate | Digital flow meters | Detect leaks in system |
| Leaf Water Potential | Remote sensing | Identify early drought stress |

### Cost-Effectiveness and Accessibility for Smallholders

High capital and maintenance costs of precision and smart irrigation systems limit adoption among smallholders, who often lack financing and technical training, despite potential 20-50% water savings (Jumintono et al., 2024; Kushwaha et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikp1bWludG9ubyIsImZhbWlseSI6Ikp1bWludG9ubyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJOdXJuYXdhdHkiLCJmYW1pbHkiOiJOdXJuYXdhdHkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTnVncm9obyBUcmlzbnUiLCJmYW1pbHkiOiJCcmF0YSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJEaW5pIiwiZmFtaWx5IjoiUmFobWlhdGkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSGFkaSIsImZhbWlseSI6IlN1c2lsbyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBLiBBLiIsImZhbWlseSI6Ikh1c3NlbiJ9fV0sImlkcyI6eyJpc3NuIjpbIjE0MjktNzQyNiIsIjIwODMtNDUzNSJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMjQ0MjUvandsZC4yMDI0LjE1MTc5OCIsIm9wZW5hbGV4IjoiVzQ0MDU0NzcxMDAifSwidGl0bGUiOiJDb21wYXJhdGl2ZSBhbmFseXNpcyBvZiBwcmVjaXNpb24sIGRlZmljaXQsIGFuZCBkcmlwIGlycmlnYXRpb24gZm9yIHdhdGVyIGVmZmljaWVuY3kgaW4gSXJhcSIsImFic3RyYWN0IjoiU3VzdGFpbmFibGUgaXJyaWdhdGlvbiBwcmFjdGljZXMgYXJlIGNyaXRpY2FsIGZvciBhZGRyZXNzaW5nIHRoZSBjaGFsbGVuZ2VzIG9mIHdhdGVyIHNjYXJjaXR5IGFuZCBmb29kIGluc2VjdXJpdHkgaW4gdGhlIGZhY2Ugb2YgY2xpbWF0ZSBjaGFuZ2UgYW5kIGdyb3dpbmcgZ2xvYmFsIGZvb2QgZGVtYW5kLiBUaGlzIHN0dWR5IGludmVzdGlnYXRlZCB0aGUgcG90ZW50aWFsIG9mIHByZWNpc2lvbiBpcnJpZ2F0aW9uIChQSSksIGRlZmljaXQgaXJyaWdhdGlvbiAoREkpLCBhbmQgZHJpcCBpcnJpZ2F0aW9uIChEUkkpIHRlY2huaXF1ZXMgdG8gZW5oYW5jZSBjcm9wIHByb2R1Y3Rpdml0eSBhbmQgd2F0ZXIgdXNlIGVmZmljaWVuY3kgKFdVRSkgY29tcGFyZWQgdG8gY29udmVudGlvbmFsIGZsb29kIGlycmlnYXRpb24gKENGSSkgaW4gdGhyZWUgZGlzdGluY3QgYWdyb2Vjb2xvZ2ljYWwgem9uZXMgb2YgSXJhcS4gRmllbGQgZXhwZXJpbWVudHMgd2VyZSBjb25kdWN0ZWQgdXNpbmcgYSByYW5kb21pc2VkIGNvbXBsZXRlIGJsb2NrIGRlc2lnbiB3aXRoIHdoZWF0LCBtYWl6ZSwgYW5kIHJpY2UgY3JvcHMuIFRoZSByZXN1bHRzIGRlbW9uc3RyYXRlZCB0aGF0IFBJIHNpZ25pZmljYW50bHkgaW5jcmVhc2VkIGNyb3AgZ3Jvd3RoIHBhcmFtZXRlcnMsIGdyYWluIHlpZWxkLCBhbmQgV1VFIGFjcm9zcyBhbGwgem9uZXMsIHdpdGggeWllbGQgaW1wcm92ZW1lbnRzIG9mIDMz4oCTMzglIGFuZCBXVUUgaW5jcmVhc2VzIG9mIDQ24oCTNTElIGluIGNvbnRyYXN0IHdpdGggQ0ZJLiBUaGUgREkgYW5kIERSSSB0cmVhdG1lbnRzIGFsc28gb3V0cGVyZm9ybWVkIENGSSwgYWxiZWl0IHRvIGEgbGVzc2VyIGV4dGVudC4gUmVtb3RlIHNlbnNpbmctZGVyaXZlZCB2ZWdldGF0aW9uIGluZGljZXMgc3Ryb25nbHkgY29ycmVsYXRlZCB3aXRoIGNyb3AgZ3Jvd3RoIHBhcmFtZXRlcnMgYW5kIHlpZWxkLCB3aGlsZSBoeWRyb2xvZ2ljYWwgbW9kZWxsaW5nIHJldmVhbGVkIHJlZHVjZWQgZXZhcG90cmFuc3BpcmF0aW9uIGFuZCBzdXJmYWNlIHJ1bm9mZiB1bmRlciB0aGUgUEkgdHJlYXRtZW50LiBUaGUgc3VzdGFpbmFibGUgaXJyaWdhdGlvbiBwcmFjdGljZXMgcmVzdWx0ZWQgaW4gc3Vic3RhbnRpYWwgd2F0ZXIgc2F2aW5ncyBvZiAyMOKAkzMwJSBjb21wYXJlZCB0byBDRkkuIFRoZXNlIGZpbmRpbmdzIGhpZ2hsaWdodCB0aGUgaW1wb3J0YW5jZSBvZiBhZG9wdGluZyBlZmZpY2llbnQgaXJyaWdhdGlvbiB0ZWNobmlxdWVzLCBhbG9uZyB3aXRoIGEgaG9saXN0aWMgYXBwcm9hY2ggZW5jb21wYXNzaW5nIHRlY2hub2xvZ2ljYWwgaW5ub3ZhdGlvbnMsIGNhcGFjaXR5IGJ1aWxkaW5nLCBhbmQgc3Rha2Vob2xkZXIgZW5nYWdlbWVudCwgdG8gcHJvbW90ZSB3YXRlci0gZWZmaWNpZW50IGFncmljdWx0dXJlIGFuZCBlbnN1cmUgZm9vZCBzZWN1cml0eSBpbiB3YXRlci1zY2FyY2UgcmVnaW9ucy4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjQ0MjUvandsZC4yMDI0LjE1MTc5OCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjI0NDI1L2p3bGQuMjAyNC4xNTE3OTgiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMi0xNCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkpvdXJuYWwgb2YgV2F0ZXIgYW5kIExhbmQgRGV2ZWxvcG1lbnQiLCJpbXBhY3RGYWN0b3IiOiIxLjQ2In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTMwLCJsYXN0UGFnZSI6MTM2fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkRlIEdydXl0ZXIgT3BlbiJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MzIsImNpdGVkQnlDb3VudCI6MX19LCJxdW90ZSI6IkluIHRoaXMgY29udGV4dCwgdGhlIGFkb3B0aW9uIG9mICBzdXN0YWluYWJsZSBpcnJpZ2F0aW9uIHByYWN0aWNlcyBoYXMgZW1lcmdlZCBhcyBhIGNyaXRpY2FsIHN0cmF0ZWd5ICBmb3IgZW5oYW5jaW5nIGZvb2Qgc2VjdXJpdHkgd2hpbGUgcHJvbW90aW5nIHdhdGVyIGNvbnNlcnZhdGlvbiAgaW4gYWdyaWN1bHR1cmFsIGxhbmRzY2FwZXMuICBJcnJpZ2F0aW9uIHBsYXlzIGEgdml0YWwgcm9sZSBpbiBnbG9iYWwgZm9vZCBwcm9kdWN0aW9uLCB3aXRoICBpcnJpZ2F0ZWQgYWdyaWN1bHR1cmUgYWNjb3VudGluZyBmb3IgYXBwcm94aW1hdGVseSA0MCUgb2YgdGhlICBmb29kIHN1cHBseSAoV3JhY2hpZW4gZGUsIFNjaHVsdHogYW5kIEdvbGksIDIwMjE7IFNjaG1pdHQsIFJvc2EgIGFuZCBEYWlseSwgMjAyMikuIEhvd2V2ZXIsIHRyYWRpdGlvbmFsIGlycmlnYXRpb24gbWV0aG9kcywgc3VjaCBhcyAgZmxvb2QgYW5kIGZ1cnJvdyBpcnJpZ2F0aW9uLCBhcmUgb2Z0ZW4gaW5lZmZpY2llbnQsIGxlYWRpbmcgdG8gIHN1YnN0YW50aWFsIHdhdGVyIGxvc3NlcyB0aHJvdWdoIGV2YXBvcmF0aW9uLCBydW5vZmYsIGFuZCBkZWVwICBwZXJjb2xhdGlvbiAoQW1ib21zYSwgMjAyMCkuIE1vcmVvdmVyLCB0aGUgZXhjZXNzaXZlIGFwcGxpY2EtdGlvbiBvZiB3YXRlciBjYW4gcmVzdWx0IGluIHNvaWwgc2FsaW5pc2F0aW9uLCBudXRyaWVudCBsZWFjaGluZywgYW5kICBncm91bmR3YXRlciBkZXBsZXRpb24sIGZ1cnRoZXIgY29tcHJvbWlzaW5nIHRoZSBsb25nLXRlcm0gIHN1c3RhaW5hYmlsaXR5IG9mIGFncmljdWx0dXJhbCBzeXN0ZW1zIChZaW4gZXQgYWwuLCAyMDIyKS4gVG8gYWRkcmVzcyAgdGhlc2UgY2hhbGxlbmdlcywgcmVzZWFyY2hlcnMgYW5kIHByYWN0aXRpb25lcnMgaGF2ZSBiZWVuIGV4cGxvci1pbmcgaW5ub3ZhdGl2ZSBpcnJpZ2F0aW9uIHRlY2huaXF1ZXMgYW5kIHdhdGVyIG1hbmFnZW1lbnQgIHN0cmF0ZWdpZXMgdGhhdCBjYW4gb3B0aW1pc2Ugd2F0ZXIgdXNlIGVmZmljaWVuY3kgKFdVRSkgd2hpbGUgIG1haW50YWluaW5nIG9yIGVuaGFuY2luZyBjcm9wIHByb2R1Y3Rpdml0eS4gIFByZWNpc2lvbiBpcnJpZ2F0aW9uIChQSSkgb2ZmZXJzIGEgcHJvbWlzaW5nIHNvbHV0aW9uLCAgaW52b2x2aW5nIHRoZSB0YXJnZXRlZCBhcHBsaWNhdGlvbiBvZiB3YXRlciBzcGVjaWZpY2FsbHkgdG8gdGhlICBuZWVkcyBvZiBpbmRpdmlkdWFsIHBsYW50cyBvciBjcm9wIHpvbmVzIChaaW5rZXJuYWdlbCBldCBhbC4sIDIwMjApLiAgIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyMi44ODY3MzU5MTYxMzc2OTUsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzQ2MjIxOTIzODI4MTI1LCJoZWlnaHQiOjEuNDUwNTQxMDE5NDM5Njk3M319LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI0LjM1MTEyMzgwOTgxNDQ1MywibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTI3MTYwNjQ0NSwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI1LjcyNDk4NzAzMDAyOTI5NywibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTY1MzA3NjE3MiwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI3LjA5ODg1MDI1MDI0NDE0LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjE2LjQxODE2MTM5MjIxMTkxNCwiaGVpZ2h0IjoxLjM2MDAxNDc5NjI1NzAxOX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI4LjQ3MjcwOTY1NTc2MTcyLCJsZWZ0IjoxMi4zODA4ODAzNTU4MzQ5NjEsIndpZHRoIjozNy4wMTIxODQxNDMwNjY0MDYsImhlaWdodCI6MS4zNjAwMTQ3OTYyNTcwMTl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyOS44NDY1NzI4NzU5NzY1NjIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQwNjQ4NjUxMTIzMDUsImhlaWdodCI6MS4zNjAwMTY3MDM2MDU2NTE5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzEuMjE0MDM2OTQxNTI4MzIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQ0ODU2MjYyMjA3MDMsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzIuNTg3OTAyMDY5MDkxOCwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDYwNTc4OTE4NDU3LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMzLjk2MTc2MTQ3NDYwOTM3NSwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDA0ODQ2MTkxNDA2MjUsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzUuMzM1NjI0Njk0ODI0MjIsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzM5NzQ0NTY3ODcxMDk0LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjM2LjcwOTQ4NzkxNTAzOTA2LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjAwMTU5MDcyODc1OTc2NiwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC4wODMzNDczMjA1NTY2NCwibGVmdCI6OS4wNDc2MDI2NTM1MDM0MTgsIndpZHRoIjo0MC4zNDU3NTY1MzA3NjE3MiwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOS40NTcyMTA1NDA3NzE0ODQsImxlZnQiOjkuMDQ3NjAyNjUzNTAzNDE4LCJ3aWR0aCI6NDAuMzQzMDQ4MDk1NzAzMTI1LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQwLjc0MDU0NzE4MDE3NTc4LCJsZWZ0Ijo5LjA0NzYwMjY1MzUwMzQxOCwid2lkdGgiOjQwLjM0NTQ1NTE2OTY3NzczNCwiaGVpZ2h0IjoxLjQ1MDU0MjgwNzU3OTA0MDV9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Mi4yMDQ5MzMxNjY1MDM5MDYsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMDAzMjUwMTIyMDcwMzEsImhlaWdodCI6MS4zNjAwMTg0OTE3NDQ5OTUxfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDMuNTc4Nzk2Mzg2NzE4NzUsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzM3OTIxMTQyNTc4MTI1LCJoZWlnaHQiOjEuMzYwMDE4NDkxNzQ0OTk1MX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQ0Ljg2MjEzMzAyNjEyMzA1LCJsZWZ0Ijo5LjA0NzYwMTY5OTgyOTEwMiwid2lkdGgiOjQwLjM0NTMxNDAyNTg3ODkwNiwiaGVpZ2h0IjoxLjQ1MDU0MjgwNzU3OTA0MDV9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ni4zMjY1MTkwMTI0NTExNywibGVmdCI6OS4wNDc2MDE2OTk4MjkxMDIsIndpZHRoIjoyNy45Njc0NDcyODA4ODM3OSwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ny43MDAzODIyMzI2NjYwMTYsImxlZnQiOjEyLjM4MDg3ODQ0ODQ4NjMyOCwid2lkdGgiOjM3LjAzNDc4NjIyNDM2NTIzNCwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0OS4wNzQyNDU0NTI4ODA4NiwibGVmdCI6OS4wNDc2MDE2OTk4MjkxMDIsIndpZHRoIjo0MC4zNDA2NDQ4MzY0MjU3OCwiaGVpZ2h0IjoxLjM2MDAxODQ5MTc0NDk5NTF9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1MC4zNTc1ODIwOTIyODUxNTYsImxlZnQiOjkuMDQ3NjAxNjk5ODI5MTAyLCJ3aWR0aCI6NDAuMzQ0ODUyNDQ3NTA5NzY2LCJoZWlnaHQiOjEuNDUwNTQyODA3NTc5MDQwNX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEzMSJ9fSx7Im9yaWdpbiI6NCwid29yayI6eyJ0eXBlIjozMCwic3VidHlwZSI6e30sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1hbmlzaCIsImZhbWlseSI6Ikt1c2h3YWhhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNoYW5rYXIiLCJmYW1pbHkiOiJTaW5naCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJWaWpheSBLYW50IiwiZmFtaWx5IjoiU2luZ2gifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2hhc2hhbmsiLCJmYW1pbHkiOiJEd2l2ZWRpIn19XSwiaWRzIjp7Imlzc24iOlsiMjQ1Ni0xODc4Il0sImlzYm4iOltdLCJkb2kiOiIxMC4yMjE2MS9pamVhYi45Mi4yNyIsIm9wZW5hbGV4IjoiVzQzOTY1NjM1ODQifSwidGl0bGUiOiJQcmVjaXNpb24gRmFybWluZzogQSBSZXZpZXcgb2YgTWV0aG9kcywgVGVjaG5vbG9naWVzLCBhbmQgRnV0dXJlIFByb3NwZWN0cyIsImFic3RyYWN0IjoiUHJlY2lzaW9uIGZhcm1pbmcsIG9mdGVuIHJlZmVycmVkIHRvIGFzIHByZWNpc2lvbiBhZ3JpY3VsdHVyZSwgaXMgYSBmYXJtaW5nIG1hbmFnZW1lbnQgY29uY2VwdCB0aGF0IHNlZWtzIHRvIG1heGltaXplIGFncmljdWx0dXJhbCBvcGVyYXRpb25zIHZpYSB0aGUgdXNlIG9mIHRlY2hub2xvZ3kgaW4gb3JkZXIgdG8gYWNoaWV2ZSBncmVhdGVyIGxldmVscyBvZiBlZmZpY2llbmN5LCBwcm9kdWN0aW9uLCBhbmQgY29uc2VydmF0aW9uIG9mIHJlc291cmNlcy4gVGhlIHB1cnBvc2Ugb2YgdGhpcyBzdXJ2ZXkgc3R1ZHkgaXMgdG8gb2ZmZXIgYW4gb3ZlcnZpZXcgb2YgcHJlY2lzaW9uIGZhcm1pbmcsIGluY2x1ZGluZyB0b3BpY3Mgc3VjaCBhcyBpdHMgZnVuZGFtZW50YWxzLCBtZXRob2RvbG9naWVzLCB0ZWNobm9sb2d5LCBhbmQgYXBwbGljYXRpb25zLiBJdCBpbnZlc3RpZ2F0ZXMgdGhlIHJvbGUgdGhhdCBhIHZhcmlldHkgb2YgdGVjaG5vbG9naWVzLCBpbmNsdWRpbmcgZ2xvYmFsIG5hdmlnYXRpb24gc2F0ZWxsaXRlIHN5c3RlbXMgKEdOU1MpLCBnZW9ncmFwaGljIGluZm9ybWF0aW9uIHN5c3RlbXMgKEdJUyksIHJlbW90ZSBzZW5zaW5nLCBhbmQgc2Vuc29yIHRlY2hub2xvZ3ksIHBsYXkgaW4gbWFraW5nIGl0IHBvc3NpYmxlIHRvIGltcGxlbWVudCBwcmVjaXNpb24gYWdyaWN1bHR1cmFsIG1ldGhvZHMuIEluIGFkZGl0aW9uLCBpdCBhZGRyZXNzZXMgdGhlIGFkdmFudGFnZXMsIGRpc2FkdmFudGFnZXMsIGFuZCBwb3RlbnRpYWwgZnV0dXJlIGFwcGxpY2F0aW9ucyBvZiBwcmVjaXNpb24gZmFybWluZyBpbiB0aGUgY29udGV4dCBvZiBzb2x2aW5nIGlzc3VlcyByZWxhdGVkIHRvIHRoZSBzdXN0YWluYWJpbGl0eSBvZiBhZ3JpY3VsdHVyZSBhbmQgdGhlIHNlY3VyaXR5IG9mIGZvb2Qgc3VwcGxpZXMgb24gYSB3b3JsZHdpZGUgc2NhbGUuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjIyMTYxL2lqZWFiLjkyLjI3IiwicmVzb3VyY2VVcmwiOiJodHRwczovL2lqZWFiLmNvbS91cGxvYWRfZG9jdW1lbnQvaXNzdWVfZmlsZXMvMjdJSkVBQi0xMDQyMDI0ODQtUHJlY2lzaW9uRmFybWluZy5wZGYiLCJpc3N1ZWREYXRlIjoiMjAyNC0wMS0wMSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkludGVybmF0aW9uYWwgSm91cm5hbCBvZiBFbnZpcm9ubWVudCBBZ3JpY3VsdHVyZSBhbmQgQmlvdGVjaG5vbG9neSIsImlzc3VlIjoyLCJ2b2x1bWUiOjksImltcGFjdEZhY3RvciI6IjAuNjUifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjoyNDIsImxhc3RQYWdlIjoyNTN9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiSW5mb2dhaW4gUHVibGljYXRpb24ifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjQsImNpdGVkQnlDb3VudCI6OH19LCJxdW90ZSI6IklJSS4gQkVORUZJVFMgT0YgUFJFQ0lTSU9OIElSUklHQVRJT04gMS4gSW1wcm92ZWQgV2F0ZXIgVXNlIEVmZmljaWVuY3k6IFByZWNpc2lvbiBpcnJpZ2F0aW9uIG9wdGltaXplcyB3YXRlciB1c2UgZWZmaWNpZW5jeSBieSBkZWxpdmVyaW5nIHdhdGVyIGRpcmVjdGx5IHRvIHRoZSByb290IHpvbmUgb2YgY3JvcHMsIG1pbmltaXppbmcgbG9zc2VzICBkdWUgdG8gZXZhcG9yYXRpb24sIHJ1bm9mZiwgYW5kIGRlZXAgcGVyY29sYXRpb24uIEJ5ICBtYXRjaGluZyB3YXRlciBhcHBsaWNhdGlvbiByYXRlcyB0byBjcm9wIHdhdGVyICByZXF1aXJlbWVudHMsIHByZWNpc2lvbiBpcnJpZ2F0aW9uIHN5c3RlbXMgZW5zdXJlIHRoYXQgIHdhdGVyIGlzIHVzZWQgbW9yZSBlZmZpY2llbnRseSwgbWF4aW1pemluZyBjcm9wIHlpZWxkcyAgcGVyIHVuaXQgb2Ygd2F0ZXIgYXBwbGllZC4gMi4gRW5oYW5jZWQgQ3JvcCBQZXJmb3JtYW5jZTogQnkgcHJvdmlkaW5nIGNyb3BzIHdpdGggb3B0aW1hbCBtb2lzdHVyZSBsZXZlbHMsIHByZWNpc2lvbiBpcnJpZ2F0aW9uIHN5c3RlbXMgcHJvbW90ZSBoZWFsdGh5IHJvb3QgZGV2ZWxvcG1lbnQsIGJhbGFuY2VkIGdyb3d0aCwgIGFuZCBpbXByb3ZlZCBjcm9wIHZpZ29yLiBDb25zaXN0ZW50IHNvaWwgbW9pc3R1cmUgIGxldmVscyBoZWxwIHJlZHVjZSB3YXRlciBzdHJlc3MsIGluY3JlYXNlIG51dHJpZW50ICB1cHRha2UsIGFuZCBlbmhhbmNlIGNyb3AgcmVzaWxpZW5jZSB0byBlbnZpcm9ubWVudGFsICBzdHJlc3NlcyBzdWNoIGFzIGRyb3VnaHQgYW5kIGhlYXQuIDMuIENvc3QgU2F2aW5nczogUHJlY2lzaW9uIGlycmlnYXRpb24gc3lzdGVtcyBoZWxwIGZhcm1lcnMgcmVkdWNlIHdhdGVyLCBlbmVyZ3ksIGFuZCBsYWJvciBjb3N0cyBhc3NvY2lhdGVkIHdpdGggaXJyaWdhdGlvbi4gQnkgb3B0aW1pemluZyB3YXRlciB1c2UgZWZmaWNpZW5jeSBhbmQgIG1pbmltaXppbmcgaW5wdXQgd2FzdGFnZSwgcHJlY2lzaW9uIGlycmlnYXRpb24gc3lzdGVtcyAgY2FuIGxlYWQgdG8gc2lnbmlmaWNhbnQgY29zdCBzYXZpbmdzIG92ZXIgdGltZSwgIGltcHJvdmluZyB0aGUgcHJvZml0YWJpbGl0eSBvZiBhZ3JpY3VsdHVyYWwgb3BlcmF0aW9ucy4gNC4gRW52aXJvbm1lbnRhbCBTdXN0YWluYWJpbGl0eTogIFByZWNpc2lvbiAgaXJyaWdhdGlvbiBjb250cmlidXRlcyB0byAgZW52aXJvbm1lbnRhbCAgc3VzdGFpbmFiaWxpdHkgIGJ5IGNvbnNlcnZpbmcgd2F0ZXIgIHJlc291cmNlcywgIG1pbmltaXppbmcgIHdhdGVyIHBvbGx1dGlvbiwgYW5kIHJlZHVjaW5nIGdyZWVuaG91c2UgZ2FzIGVtaXNzaW9ucy4gQnkgYWRvcHRpbmcgcHJlY2lzaW9uIGlycmlnYXRpb24gcHJhY3RpY2VzLCBmYXJtZXJzIGNhbiAgbWluaW1pemUgdGhlaXIgZW52aXJvbm1lbnRhbCBmb290cHJpbnQgYW5kIGNvbnRyaWJ1dGUgdG8gdGhlICBsb25nLXRlcm0gIHN1c3RhaW5hYmlsaXR5ICBvZiAgYWdyaWN1bHR1cmFsIHByb2R1Y3Rpb24gc3lzdGVtcy4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo0NC44NjU2NDI1NDc2MDc0MiwibGVmdCI6NTMuNDA4MjUyNzE2MDY0NDUsIndpZHRoIjozOC4yNjE3NjgzNDEwNjQ0NSwiaGVpZ2h0IjoxLjY2OTIyNzM2MTY3OTA3NzF9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo0Ni44OTc5MjI1MTU4NjkxNCwibGVmdCI6NTIuNDIwNTQzNjcwNjU0Mywid2lkdGgiOjQwLjI0MDczNzkxNTAzOTA2LCJoZWlnaHQiOjEuNjI1NDU0NzgzNDM5NjM2Mn19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjQ4LjQ5MTc5MDc3MTQ4NDM3NSwibGVmdCI6NTQuNjk4MzEwODUyMDUwNzgsIndpZHRoIjozNy45ODAyNzgwMTUxMzY3MiwiaGVpZ2h0IjoxMS4wMTM3MTM4MzY2Njk5MjJ9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo1OS45Mzk1NDA4NjMwMzcxMSwibGVmdCI6NTIuNDIwNTQzNjcwNjU0Mywid2lkdGgiOjQwLjIzMDIyNDYwOTM3NSwiaGVpZ2h0IjoxLjYyNTQ1NDc4MzQzOTYzNjJ9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo2MS41MzM0MTY3NDgwNDY4NzUsImxlZnQiOjU0LjY5ODMxMDg1MjA1MDc4LCJ3aWR0aCI6MzcuOTYzMTIzMzIxNTMzMiwiaGVpZ2h0Ijo5LjQzNTE3MDE3MzY0NTAyfX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NzEuNDE2ODc3NzQ2NTgyMDMsImxlZnQiOjUyLjQyMDU0MzY3MDY1NDMsIndpZHRoIjo0MC4yMTMzMTAyNDE2OTkyMiwiaGVpZ2h0IjoxLjYyNTQ1ODM1OTcxODMyMjh9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo3My4wMTA3NTc0NDYyODkwNiwibGVmdCI6NTQuNjk4MzEwODUyMDUwNzgsIndpZHRoIjozNy45NDU3ODU1MjI0NjA5NCwiaGVpZ2h0Ijo3Ljg2Mzc2NDI4NjA0MTI2fX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6ODEuMzA4NTU1NjAzMDI3MzQsImxlZnQiOjUyLjQyMDU0MzY3MDY1NDMsIndpZHRoIjo0MC4yNDAzNzU1MTg3OTg4MywiaGVpZ2h0IjoxLjYyNTQ1ODM1OTcxODMyMjh9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo4Mi45MTkwNTk3NTM0MTc5NywibGVmdCI6NTQuNjk4MzEwODUyMDUwNzgsIndpZHRoIjozNy45MzY0MTY2MjU5NzY1NiwiaGVpZ2h0IjoxLjU3ODEzMzU4MzA2ODg0Nzd9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo4NC40ODY5MDAzMjk1ODk4NCwibGVmdCI6NTQuNjk4MzEwODUyMDUwNzgsIndpZHRoIjozNy45MzQ3NTcyMzI2NjYwMTYsImhlaWdodCI6MS41NzgxMzM1ODMwNjg4NDc3fX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6ODYuMDU0NzQ4NTM1MTU2MjUsImxlZnQiOjU0LjY5ODMxMDg1MjA1MDc4LCJ3aWR0aCI6MzcuOTc4NDMxNzAxNjYwMTU2LCJoZWlnaHQiOjQuNzEzODE5MDI2OTQ3MDIxNX19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjguNDIwMjkyODU0MzA5MDgyLCJsZWZ0IjoxMC4wMjIxNzI5Mjc4NTY0NDUsIndpZHRoIjozNy45NDY4MzA3NDk1MTE3MiwiaGVpZ2h0IjoxLjU3ODEzNTI1MTk5ODkwMTR9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo5Ljk4ODEzMjQ3NjgwNjY0LCJsZWZ0IjoxMC4wMjIxNzI5Mjc4NTY0NDUsIndpZHRoIjoxMy44NjI0NjEwOTAwODc4OSwiaGVpZ2h0IjoxLjU3ODEzNTI1MTk5ODkwMTR9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIyNDkifX1d. Subsidies, microfinance, and modular designs are essential, yet equitable access remains challenged by patented technologies and supply chain issues in developing regions (Xie, 2025)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjMwLCJzdWJ0eXBlIjp7fSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQ2hlbiIsImZhbWlseSI6IlhpZSJ9fV0sImlkcyI6eyJpc3NuIjpbIjE2NjQtNDYyWCJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMzM4OS9mcGxzLjIwMjUuMTY3NTY1NyIsInBtaWQiOiI0MTAzNjM5MyIsIm9wZW5hbGV4IjoiVzQ0MTQyNDgzMDcifSwidGl0bGUiOiJUaGUgcm9sZSBvZiBtb2Rlcm4gYWdyaWN1bHR1cmFsIHRlY2hub2xvZ2llcyBpbiBpbXByb3ZpbmcgYWdyaWN1bHR1cmFsIHByb2R1Y3Rpdml0eSBhbmQgbGFuZCB1c2UgZWZmaWNpZW5jeSIsImFic3RyYWN0IjoiTW9kZXJuIGFncmljdWx0dXJhbCB0ZWNobm9sb2dpZXMgYXJlIGNydWNpYWwgZm9yIGFkZHJlc3NpbmcgZ2xvYmFsIGZvb2Qgc2VjdXJpdHkgYW5kIGVudmlyb25tZW50YWwgc3VzdGFpbmFiaWxpdHkgY2hhbGxlbmdlcyBhbWlkc3QgYSBncm93aW5nIHBvcHVsYXRpb24gYW5kIGNsaW1hdGUgY2hhbmdlLiBUaGVzZSBpbm5vdmF0aW9ucywgaW5jbHVkaW5nIHByZWNpc2lvbiBhZ3JpY3VsdHVyZSwgYmlvdGVjaG5vbG9neSwgc21hcnQgaXJyaWdhdGlvbiwgYXV0b21hdGlvbiwgdmVydGljYWwgZmFybWluZywgYW5kIGFydGlmaWNpYWwgaW50ZWxsaWdlbmNlIChBSSksIHNpZ25pZmljYW50bHkgZW5oYW5jZSBwcm9kdWN0aXZpdHkgYW5kIGxhbmQgdXNlIGVmZmljaWVuY3kuIFByZWNpc2lvbiBhZ3JpY3VsdHVyZSwgdXRpbGl6aW5nIEdQUywgZHJvbmVzLCBhbmQgSW9ULCBpbXByb3ZlcyB5aWVsZHMgYnkgMjDigJMzMCUgYW5kIGN1dHMgaW5wdXQgd2FzdGUgYnkgNDDigJM2MCUuIEJpb3RlY2hub2xvZ3ksIHdpdGggQ1JJU1BSIGFuZCBHTU9zLCBkZWxpdmVycyBkcm91Z2h0IGFuZCBwZXN0LXJlc2lzdGFudCBjcm9wcywgc3RhYmlsaXppbmcgeWllbGRzLCBhcyBzZWVuIHdpdGggQnQgY290dG9uIHJlZHVjaW5nIHBlc3RpY2lkZSB1c2UgYnkgNTAlIGluIEluZGlhLiBTbWFydCBpcnJpZ2F0aW9uIGJvb3N0cyB3YXRlciBlZmZpY2llbmN5IGJ5IDQw4oCTNjAlLCB3aGlsZSBhdXRvbWF0aW9uIGFuZCByb2JvdGljcyBtaXRpZ2F0ZSBsYWJvciBzaG9ydGFnZXMgYW5kIHJlZHVjZSBjb3N0cyBieSAyNSUuIFZlcnRpY2FsIGZhcm1pbmcgaW5jcmVhc2VzIHlpZWxkcyAxMOKAkzIwIHRpbWVzIHdpdGggOTUlIGxlc3MgbGFuZCBhbmQgd2F0ZXIsIHN1cHBvcnRpbmcgdXJiYW4gZm9vZCBzZWN1cml0eS4gQUkgYW5hbHl0aWNzIGVuaGFuY2UgZGVjaXNpb24tbWFraW5nIHdpdGggb3ZlciA5MCUgYWNjdXJhY3kgaW4gZm9yZWNhc3RpbmcgYW5kIHJlc291cmNlIGFsbG9jYXRpb24uIERlc3BpdGUgdGhlc2UgYmVuZWZpdHMsIGhpZ2ggY29zdHMsIHRlY2hub2xvZ2ljYWwgaWxsaXRlcmFjeSwgYW5kIHJlZ3VsYXRvcnkgaXNzdWVzIGhpbmRlciBhZG9wdGlvbiwgZXNwZWNpYWxseSBhbW9uZyBzbWFsbGhvbGRlcnMuIFBvbGljeSBzdXBwb3J0LCBwdWJsaWMtcHJpdmF0ZSBwYXJ0bmVyc2hpcHMsIGFuZCB0cmFpbmluZyBhcmUgdml0YWwgZm9yIGJyb2FkZXIgdGVjaG5vbG9neSBhY2Nlc3MgYW5kIGZhaXIgYmVuZWZpdHMuIEludGVncmF0aW5nIHJlbmV3YWJsZSBlbmVyZ3kgYW5kIGNpcmN1bGFyIGVjb25vbXkgcHJpbmNpcGxlcyBpbnRvIGFnZ3JvdGVjaCBwcmVzZW50cyBhIHBhdGggdG8gc3VzdGFpbmFiaWxpdHkuIFRoaXMgcmV2aWV3IGhpZ2hsaWdodHMgdGhlIHRyYW5zZm9ybWF0aXZlIHBvdGVudGlhbCBvZiBtb2Rlcm4gdGVjaG5vbG9naWVzIGZvciBzdXN0YWluYWJsZSBpbnRlbnNpZmljYXRpb24sIGluY3JlYXNpbmcgcHJvZHVjdGl2aXR5IHdpdGhvdXQgZXhwYW5kaW5nIGZhcm1sYW5kLCB3aGlsZSBsZXNzZW5pbmcgZW52aXJvbm1lbnRhbCBpbXBhY3RzLiBJdCB1bmRlcnNjb3JlcyB0aGUgbmVlZCBmb3IgY29vcmRpbmF0ZWQgZWZmb3J0cyB0byBvdmVyY29tZSBhZG9wdGlvbiBjaGFsbGVuZ2VzIGFuZCBoYXJuZXNzIHRoZXNlIGlubm92YXRpb25zIGZvciBnbG9iYWwgZm9vZCBzZWN1cml0eSBhbmQgY2xpbWF0ZSByZXNpbGllbmNlLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzg5L2ZwbHMuMjAyNS4xNjc1NjU3IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5mcm9udGllcnNpbi5vcmcvam91cm5hbHMvcGxhbnQtc2NpZW5jZS9hcnRpY2xlcy8xMC4zMzg5L2ZwbHMuMjAyNS4xNjc1NjU3L3BkZiIsImlzc3VlZERhdGUiOiIyMDI1LTA5LTE2IiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiRnJvbnRpZXJzIGluIFBsYW50IFNjaWVuY2UiLCJ2b2x1bWUiOjE2LCJpbXBhY3RGYWN0b3IiOiI3LjAwIn0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkZyb250aWVycyBNZWRpYSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6ODgsImNpdGVkQnlDb3VudCI6MH19LCJxdW90ZSI6IlN5bnRoZXRpYyBiaW9sb2d5IGlzIGFsc28gY29udHJpYnV0aW5nLCB3aXRoIG1pY3JvYmlhbCBjb25zb3J0aWEgZW5naW5lZXJlZCB0byBlbmhhbmNlIHNvaWwgaGVhbHRoIGFuZCBjcm9wIHJlc2lsaWVuY2UgKExhd3NvbiBldCBhbC4sIDIwMTkpLiBEZXNwaXRlIHRoZXNlIGFkdmFuY2VzLCBlcXVpdGFibGUgYWNjZXNzIHJlbWFpbnMgYSBjaGFsbGVuZ2UsIGFzIHNtYWxsaG9sZGVyIGZhcm1lcnMgaW4gZGV2ZWxvcGluZyByZWdpb25zIG9mdGVuIGxhY2sgYWNjZXNzIHRvIHBhdGVudGVkIHRlY2hub2xvZ2llcyAoVG9lbm5pZXNzZW4gZXQgYWwuLCAyMDA4KS4gSW5pdGlhdGl2ZXMgbGlrZSB0aGUgT3BlbkNSSVNQUiBwcm9qZWN0IGFpbSB0byBkZW1vY3JhdGl6ZSBnZW5lLWVkaXRpbmcgdG9vbHMgKENvaGVuLCAyMDIzKSwgd2hpbGUgcHVibGljLXNlY3RvciBwYXJ0bmVyc2hpcHMgKGUuZy4sIENJTU1ZVOKAmXMgZHJvdWdodC10b2xlcmFudCBtYWl6ZSkgZW5zdXJlIGFmZm9yZGFiaWxpdHkuIEFzIGNsaW1hdGUgY2hhbmdlIGludGVuc2lmaWVzLCB0aGVzZSBiaW90ZWNobm9sb2dpZXMgd2lsbCBiZSBpbmRpc3BlbnNhYmxlIGZvciBhY2hpZXZpbmcgc3VzdGFpbmFibGUgaW50ZW5zaWZpY2F0aW9uLCBidXQgdGhlaXIgc3VjY2VzcyBoaW5nZXMgb24gZ2xvYmFsIGNvbGxhYm9yYXRpb24sIGV0aGljYWwgZ292ZXJuYW5jZSwgYW5kIGluY2x1c2l2ZSBpbm5vdmF0aW9uLiAzLjMgU21hcnQgaXJyaWdhdGlvbiAmIHdhdGVyIG1hbmFnZW1lbnQgV2F0ZXIgc2NhcmNpdHkgcG9zZXMgb25lIG9mIHRoZSBtb3N0IHNpZ25pZmljYW50IGNoYWxsZW5nZXMgdG8gZ2xvYmFsIGFncmljdWx0dXJlLCB3aXRoIDcwJSBvZiBmcmVzaHdhdGVyIHdpdGhkcmF3YWxzIGFsbG9jYXRlZCB0byBpcnJpZ2F0aW9uLCBvZnRlbiBpbmVmZmljaWVudGx5IChGQU8sIDIwMjApLiBTbWFydCBpcnJpZ2F0aW9uIHN5c3RlbXMsIGludGVncmF0aW5nIElvVC1iYXNlZCBzZW5zb3JzIGFuZCBkcmlwIGlycmlnYXRpb24gdGVjaG5vbG9naWVzLCBhcmUgdHJhbnNmb3JtaW5nIHdhdGVyIG1hbmFnZW1lbnQgYnkgb3B0aW1pemluZyB1c2FnZSwgcmVkdWNpbmcgd2FzdGUsIGFuZCBlbmhhbmNpbmcgY3JvcCB5aWVsZHMuIFNvaWwgbW9pc3R1cmUgc2Vuc29ycywgc3VjaCBhcyBjYXBhY2l0aXZlIHByb2JlcyBhbmQgdGVuc2lvbWV0ZXJzLCBwcm92aWRlIHJlYWwtdGltZSBkYXRhIG9uIHdhdGVyIGF2YWlsYWJpbGl0eSBhdCB2YXJ5aW5nIHJvb3QgZGVwdGhzLCBlbmFibGluZyBwcmVjaXNlIGlycmlnYXRpb24gc2NoZWR1bGluZy4gVGhlc2Ugc2Vuc29ycyB0cmFuc21pdCBkYXRhIHZpYSBMb1JhV0FOIG9yIGNlbGx1bGFyIG5ldHdvcmtzIHRvIGNsb3VkIHBsYXRmb3Jtcywgd2hlcmUgQUkgYWxnb3JpdGhtcyBhbmFseXplIHRyZW5kcyBhbmQgYXV0b21hdGUgaXJyaWdhdGlvbiBkZWNpc2lvbnMgKEdvYXAgZXQgYWwuLCAyMDE4KS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo1Ny4xNzU2NDM5MjA4OTg0NCwibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ5MzE3NTUwNjU5MTgsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NTguNjY0MjM0MTYxMzc2OTUsImxlZnQiOjguMzMxNDQyODMyOTQ2Nzc3LCJ3aWR0aCI6NDAuNDkzOTUzNzA0ODMzOTg0LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjYwLjE0NjA4MDAxNzA4OTg0NCwibGVmdCI6OC4zMzE0NDE4NzkyNzI0NjEsIndpZHRoIjo0MC40OTc1ODE0ODE5MzM1OTQsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NjEuNjM0Njc3ODg2OTYyODksImxlZnQiOjguMzMxNDQxODc5MjcyNDYxLCJ3aWR0aCI6NDAuNDk1OTA2ODI5ODMzOTg0LCJoZWlnaHQiOjEuMTgwNTE2OTU4MjM2Njk0M319LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjYzLjExNjUyNzU1NzM3MzA1LCJsZWZ0Ijo4LjMzMTQ0MjgzMjk0Njc3Nywid2lkdGgiOjQwLjQ5NzU3NzY2NzIzNjMzLCJoZWlnaHQiOjEuMTgwNTE2OTU4MjM2Njk0M319LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjY0LjYwNTExNzc5Nzg1MTU2LCJsZWZ0Ijo4LjMzMTQzOTk3MTkyMzgyOCwid2lkdGgiOjQwLjQ4ODM0NjA5OTg1MzUxNiwiaGVpZ2h0IjoxLjE4MDUxNjk1ODIzNjY5NDN9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo2Ni4wODY5Njc0NjgyNjE3MiwibGVmdCI6OC4zMzE0NDE4NzkyNzI0NjEsIndpZHRoIjo0MC40ODMyMDc3MDI2MzY3MiwiaGVpZ2h0IjoxLjE4MDUxNjk1ODIzNjY5NDN9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo2Ny41NzU1NTM4OTQwNDI5NywibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ4MzA1MTMwMDA0ODgzLCJoZWlnaHQiOjEuMTgwNTE2OTU4MjM2Njk0M319LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjY5LjA1NzQwMzU2NDQ1MzEyLCJsZWZ0Ijo4LjMzMTU5MTYwNjE0MDEzNywid2lkdGgiOjQwLjQ4MTg2MTExNDUwMTk1LCJoZWlnaHQiOjEuMTgwNTE2OTU4MjM2Njk0M319LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjcwLjU0NTk5NzYxOTYyODksImxlZnQiOjguMzMxNTg5Njk4NzkxNTA0LCJ3aWR0aCI6NDAuNDg2MDgzOTg0Mzc1LCJoZWlnaHQiOjEuMTgwNTE2OTU4MjM2Njk0M319LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjcyLjAyNzg0NzI5MDAzOTA2LCJsZWZ0Ijo4LjMzMTQzOTAxODI0OTUxMiwid2lkdGgiOjM4LjIxMTg3MjEwMDgzMDA4LCJoZWlnaHQiOjEuMTgwNTE2OTU4MjM2Njk0M319LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjc2LjEzOTU0MTYyNTk3NjU2LCJsZWZ0Ijo4LjMzMTc0NzA1NTA1MzcxMSwid2lkdGgiOjM4LjQ1NjgyMTQ0MTY1MDM5LCJoZWlnaHQiOjEuNTU4MzMyMjA0ODE4NzI1Nn19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjc5LjQ1ODYyNTc5MzQ1NzAzLCJsZWZ0Ijo4LjMzMTU5OTIzNTUzNDY2OCwid2lkdGgiOjQwLjUwOTY1ODgxMzQ3NjU2LCJoZWlnaHQiOjExLjU3MzY4NjU5OTczMTQ0NX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjkuNjU1MTM2MTA4Mzk4NDM4LCJsZWZ0Ijo1MS4yMDkyMTcwNzE1MzMyLCJ3aWR0aCI6NDAuNDg1NjQ1Mjk0MTg5NDUsImhlaWdodCI6MS4xODA1MTk1ODA4NDEwNjQ1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6MTEuMTM2OTkxNTAwODU0NDkyLCJsZWZ0Ijo1MS4yMDkyMTcwNzE1MzMyLCJ3aWR0aCI6NDAuNDgzMTk2MjU4NTQ0OTIsImhlaWdodCI6MS4xODA1MTk1ODA4NDEwNjQ1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6MTIuNjI1NTgzNjQ4NjgxNjQsImxlZnQiOjUxLjIwOTIxNzA3MTUzMzIsIndpZHRoIjo0MC40OTA3NTY5ODg1MjUzOSwiaGVpZ2h0IjoxLjE4MDUxOTU4MDg0MTA2NDV9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI2In19XQ%3D%3D. Policy incentives, such as those promoting open-access initiatives, can enhance affordability and scalability for resource-poor farmers (Borin, 2024; Xie, 2025)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1hdXJpemlvIiwiZmFtaWx5IjoiQm9yaW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMDM4LTU2MjUiXSwiaXNibiI6W10sImRvaSI6IjEwLjM2MjUzL2lqYW0tMjMyMyIsIm9wZW5hbGV4IjoiVzQzOTEyMjg5NjIifSwidGl0bGUiOiJBIHdpc2UgaXJyaWdhdGlvbiB0byBjb250cmlidXRlIHRvIGludGVncmF0ZWQgd2F0ZXIgcmVzb3VyY2UgbWFuYWdlbWVudCIsImFic3RyYWN0IjoiSXJyaWdhdGVkIGFncmljdWx0dXJlIGFjY291bnRzIGZvciBhYm91dCAyMCUgb2YgY3VsdGl2YXRlZCBsYW5kcyB3b3JsZHdpZGUgd2hpbGUgY3VycmVudGx5IGdlbmVyYXRpbmcgYW4gZXN0aW1hdGVkIDQwJSBvZiBjcm9wIHByb2R1Y3Rpb24sIGFuZCB0d28tdGhpcmRzIG9mIGZ1dHVyZSBnYWlucyBpbiBjcm9wIHByb2R1Y3Rpb24gYXJlIGV4cGVjdGVkIHRvIGNvbWUgZnJvbSBpcnJpZ2F0ZWQgbGFuZHMuIFRoZXJlZm9yZSwgaXJyaWdhdGlvbiBpcyBzdHJhdGVnaWMgdG8gZW5zdXJlIGZvb2QgZm9yIHRoZSB3b3JsZOKAmXMgaW5jcmVhc2luZyBwb3B1bGF0aW9uIGFuZCBzbG93IGRvd24gdGhlIHBhY2Ugb2YgZGVmb3Jlc3RhdGlvbi4gSXJyaWdhdGVkIGFncmljdWx0dXJlIGFsc28gYWNjb3VudHMgZm9yIG1vcmUgdGhhbiA3MCUgb2YgZ2xvYmFsIHdhdGVyIHdpdGhkcmF3YWxzLCBhbmQgdGhlIHdheSBhZ3JpY3VsdHVyZSB1c2VzIGZyZXNod2F0ZXIgaXMgY3J1Y2lhbCBmb3IgZW5zdXJpbmcgYXZhaWxhYmlsaXR5IGZvciBvdGhlciBhY3Rpdml0aWVzIGFuZCBwcmVzZXJ2aW5nIHdhdGVyLXJlbGF0ZWQgZWNvc3lzdGVtcy4gVG8gZ2l2ZSBhbiBhZ3JpLWVudmlyb25tZW50YWwgaW50ZXJwcmV0YXRpb24gb2YgdGhlIHdhdGVyIGZsdXhlcyBpbnZvbHZlZCBpbiBpcnJpZ2F0aW9uLCB0aGUgdG90YWwgYW1vdW50IG9mIHdhdGVyIHdpdGhkcmF3biBmcm9tIGEgc291cmNlIGlzIGNhbGxlZCB3aGl0ZSB3YXRlciAoVykgYW5kIHN1YmRpdmlkZWQgYXMgZm9sbG93czogZ29sZCB3YXRlciAoRykgaXMgdGhlIGFtb3VudCB0aGF0IGlzIGFjdHVhbGx5IHVzZWQgYnkgdGhlIGNyb3BzLiBUaGlzIGZyYWN0aW9uIHJlcHJlc2VudHMgdGhlIHVsdGltYXRlIGdvYWwgb2YgaXJyaWdhdGlvbiwgd2hpY2ggaXMgdG8gaW5jcmVhc2UgY3JvcCBwcm9kdWN0aXZpdHkgYW5kIGZvb2QgYXZhaWxhYmlsaXR5LiBXZSBjYW4gYWxzbyBjb25zaWRlciB0aGlzIGFtb3VudCBhcyB0aGUgaXJyaWdhdGlvbiB3YXRlciBmb3IgY3JvcHMuIFRoZSBnb2xkIGNvbG9yIG1lYW5zIGFuIGluY29tZSBmb3IgdGhlIGZhcm07IGVtZXJhbGQgd2F0ZXIgKEUpIGlzIHRoZSBhbW91bnQgb2Ygd2F0ZXIgd2l0aGRyYXduIGZvciBpcnJpZ2F0aW9uIHRoYXQgZG9lcyBub3QgcmVhY2ggdGhlIGNyb3BzIGJ1dCBwcm92aWRlcyBlY29zeXN0ZW0gc2VydmljZXMsIGkuZS4sIHRoZSBiZW5lZml0cyB0aGF0IHBlb3BsZSBnZXQgZnJvbSBlY29zeXN0ZW1zLiBUaGlzIGFtb3VudCBvZiB3YXRlciBpcyB1c2VmdWwgZm9yIHRoZSBjb21tdW5pdHkgYW5kIGNhbiBiZSBpZGVudGlmaWVkIGFzIGlycmlnYXRpb24gb2YgdGhlIHRlcnJpdG9yeTsgcmVkIHdhdGVyIChSKSBpcyBsb3NzZXMsIGkuZS4sIHRoZSBmcmFjdGlvbiBvZiB3YXRlciB0aGF0IGhhcyB0byBiZSByZWR1Y2VkIGFzIG11Y2ggYXMgcG9zc2libGUgaWYgbm90IGVsaW1pbmF0ZWQuIEl0IGlzIGFuIGVjb25vbWljLCBzb2NpYWwgYW5kIGVudmlyb25tZW50YWwgY29zdDsgdGhlIHJlZCBjb2xvciBpcyBhc3NvY2lhdGVkIHdpdGggdGhlIHJlZCB0cmFmZmljIGxpZ2h0IHRoYXQgbWVhbnMgU1RPUCEgVGhlIHF1YW50aWZpY2F0aW9uIG9mIFcgaGFzIHRvIGJlIHByZWNlZGVkIGJ5IGEgY2FyZWZ1bCBldmFsdWF0aW9uIG9mIHRoZSBiZW5lZml0cyBhY2hpZXZhYmxlIHdpdGggaXJyaWdhdGlvbiBpbiByZWxhdGlvbiB0byB0aGUgZW52aXJvbm1lbnRhbCBjaGFyYWN0ZXJpc3RpY3Mgb2YgdGhlIGN1bHRpdmF0aW9uIGFyZWEuIFN0cmF0ZWdpYyBpc3N1ZXMgYXQgdGhpcyBzdGFnZSBhcmUgdGhlIGNob2ljZSBiZXR3ZWVuIGZ1bGwgb3IgZGVmaWNpdCBpcnJpZ2F0aW9uIGFuZCB0aGUgaW5kaXZpZHVhdGlvbiBvZiBvcHBvcnR1bml0aWVzIHRvIGV4cGxvcmF0aW9uIG9mIG9wcG9ydHVuaXRpZXMgdG8gaW5jcmVhc2UgdGhlIGF2YWlsYWJpbGl0eSBvZiB3YXRlciByZXNvdXJjZXMgYnkgdXNpbmcgbm9uLWNvbnZlbnRpb25hbCBzb3VyY2VzLiBUaGUgcXVhbnRpZmljYXRpb24gb2YgRyBhbmQgRSBmbHV4ZXMgaXMgbmVjZXNzYXJ5IHRvIHJlZHVjZSBsb3NzZXMsIHdoaWxlIHN0cmF0ZWdpZXMvdGVjaG5pcXVlcyB1c2VmdWwgdG8gcmVkdWNlIGlycmlnYXRpb24gbmVlZHMgYXQgdGhlIGZpZWxkIGxldmVsIGFyZSBrZXkgZmFjdG9ycyB0byBhbGxvY2F0ZSBpcnJpZ2F0aW9uIHdhdGVyIHdpdGhpbiB0aGUgZnJhbWV3b3JrIG9mIGFuIGludGVncmF0ZWQgYW5kIHN1c3RhaW5hYmxlIG1hbmFnZW1lbnQgb2Ygd2F0ZXIgYWltZWQgYXQgdHVybmluZyB1c2VyIGNvbmZsaWN0cyBpbnRvIHN5bmVyZ2llcy4gVGhpcyBpbXBsaWVzIG11bHRpcGxlIHN1YmplY3RzIGFuZCBhY3RvcnMgaW4gYSBtdWx0aS1kaXNjaXBsaW5hcnkgYXBwcm9hY2guIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjM2MjUzL2lqYW0tMjMyMyIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9yaXZpc3RlLmZ1cHJlc3MubmV0L2luZGV4LnBocC9JSkFtL2FydGljbGUvZG93bmxvYWQvMjMyMy8xNjIxIiwiaXNzdWVkRGF0ZSI6IjIwMjQtMDEtMjAiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTEwIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJJdGFsaWFuIEpvdXJuYWwgb2YgQWdyb21ldGVvcm9sb2d5IiwiaXNzdWUiOjIsImltcGFjdEZhY3RvciI6IjEuMDAifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjo1LCJsYXN0UGFnZSI6MTl9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRmlyZW56ZSBVbml2ZXJzaXR5IFByZXNzIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjozOSwiY2l0ZWRCeUNvdW50IjoyfX0sInF1b3RlIjoiUHJvbWlzaW5nIHJlc3VsdHMgYXJlICByZWxhdGVkIHRvIHRoZSBhdmFpbGFiaWxpdHkgb2YgZHJvdWdodC10b2xlcmFudCBtYWl6ZSB2YXJpLWV0aWVzIHRoYXQgeWllbGQgbW9yZSBpbiBkcm91Z2h0LXN0cmVzc2VkIGVudmlyb25tZW50cyAgd2l0aCBubyBwZW5hbHR5IGluIG5vbi1zdHJlc3NlZCBlbnZpcm9ubWVudHMgKEFkZWUgIGV0IGFsLiwgMjAxNiksIGFuZCByZXF1aXJlIGxlc3Mgd2F0ZXIgdG8gbWF4aW1pemUgZ3JhaW4gIHlpZWxkIGFzIGNvbXBhcmVkIHRvIHRoZSBjb252ZW50aW9uYWwgaHlicmlkIChNb3VuY2UgIGV0IGFsLiwgMjAxNSkuIFRoZXJlZm9yZSwgY2FyZWZ1bCBzZWxlY3Rpb24gb2YgaHlicmlkcyBjYW4gIGluY3JlYXNlIGNvcm4geWllbGQgYW5kIFdVRSB1bmRlciB3YXRlci1saW1pdGVkIGNvbi1kaXRpb25zIChIYW8gZXQgYWwuLCAyMDE1KS4gVGhlIGFkZXF1YXRlIGNob2ljZSBvZiBjcm9wICBhbmQgdmFyaWV0eSBoYXMgdG8gYmUgYWNjb21wYW5pZWQgYnkgYWxsIHRoZSBvdGhlciAgb3B0aW9ucyBwZXJtaXR0aW5nIHRvIGluY3JlYXNlIHRoZSBwcm9wb3J0aW9uIG9mIGV2YXBvLXRyYW5zcGlyYXRpb24gZmxvd2luZyBhcyBwcm9kdWN0aXZlIHRyYW5zcGlyYXRpb24gYXMgdG8gIG9idGFpbiDigJxtb3JlIGNyb3AgcGVyIGRyb3DigJ0uIEFkZXF1YXRlIHRpbWluZyBhbmQgc3BhYy1pbmcgb2Ygc293aW5nLCB3ZWVkIGFuZCBwZXN0IGNvbnRyb2wsIG11bGNoaW5nLCBhcmUgb25seSAgc29tZSBleGFtcGxlcyBvZiB0aGUgd2lkZSByYW5nZSBvZiB0aGUgb3Bwb3J0dW5pdGllcyAgdGhhdCBjYW4gYmUgYWRvcHRlZC4gNS4gQ09OQ0xVRElORyBSRU1BUktTIElycmlnYXRpb24gaXMgaW5kaXNwZW5zYWJsZSB0byBhY2hpZXZlIHRoZSBaZXJvIGh1bi1nZXIgdGFyZ2V0IG9mIHRoZSAyMDMwIFVOIEFnZW5kYSBiZWNhdXNlIGl0IGlzIGEgcG93ZXItZnVsIHRvb2wgZm9yIHN1c3RhaW5hYmxlIGludGVuc2lmaWNhdGlvbiwgYW5kIGFpbXMgdG8gcHJvLWR1Y2UgbW9yZSBwZXIgc3VyZmFjZSB1bml0LiBUYWNrbGluZyB0aGUgaXNzdWUgb2YgZW5zdXJpbmcgbW9yZSBudXRyaXRpb3VzIGZvb2QgIGZvciBhIGdyb3dpbmcgcG9wdWxhdGlvbiwgdGhlIGhpZ2hlciBwcm9kdWN0aXZpdHkgb2YgaXJyaWdhdGVkIGxhbmRzIGNhbiBzbG93IGRvd24gdGhlIHBhY2Ugb2YgZGVmb3Jlc3RhdGlvbiwgaGVuY2Ug4oCcbW9yZSBpcnJpZ2F0aW9uLCBsZXNzIGRlZm9yZXN0YXRpb27igJ0uICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NTEuMDQ1NjkyNDQzODQ3NjU2LCJsZWZ0Ijo5LjUyMzgwMzcxMDkzNzUsIndpZHRoIjozOS42NzI1MzExMjc5Mjk2OSwiaGVpZ2h0IjoxLjc2MjU4NTc1OTE2MjkwMjh9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo1Mi42MTM1OTc4Njk4NzMwNSwibGVmdCI6OS41MjM4MDM3MTA5Mzc1LCJ3aWR0aCI6MzkuMjg1OTgwMjI0NjA5Mzc1LCJoZWlnaHQiOjEuNzYyNTg1NzU5MTYyOTAyOH19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjU0LjE4MTQ5OTQ4MTIwMTE3LCJsZWZ0Ijo5LjUyMzgwMzcxMDkzNzUsIndpZHRoIjozOS42NjEyNTg2OTc1MDk3NjYsImhlaWdodCI6MS43NjI1ODU3NTkxNjI5MDI4fX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NTUuNzQ5Mzk3Mjc3ODMyMDMsImxlZnQiOjkuNTIzODAzNzEwOTM3NSwid2lkdGgiOjM5LjY2NzE0ODU5MDA4Nzg5LCJoZWlnaHQiOjEuNzYyNTg1NzU5MTYyOTAyOH19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjU3LjMxNzMwNjUxODU1NDY5LCJsZWZ0Ijo5LjUyMzgwMzcxMDkzNzUsIndpZHRoIjozOS42NzMzNTUxMDI1MzkwNiwiaGVpZ2h0IjoxLjc2MjU4NTc1OTE2MjkwMjh9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo1OC44ODUyMDQzMTUxODU1NSwibGVmdCI6OS41MjM4MDM3MTA5Mzc1LCJ3aWR0aCI6MzkuNjY3OTk5MjY3NTc4MTI1LCJoZWlnaHQiOjEuNzYyNTg1NzU5MTYyOTAyOH19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjYwLjQ1MzEwNTkyNjUxMzY3LCJsZWZ0Ijo5LjUyMzgwMzcxMDkzNzUsIndpZHRoIjozOS42NjQyOTEzODE4MzU5NCwiaGVpZ2h0IjoxLjc2MjU4NTc1OTE2MjkwMjh9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo2Mi4wMjEwMDM3MjMxNDQ1MywibGVmdCI6OS41MjM4MDM3MTA5Mzc1LCJ3aWR0aCI6MzkuMjg1OTgwMjI0NjA5Mzc1LCJoZWlnaHQiOjEuNzYyNTg1NzU5MTYyOTAyOH19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjYzLjU4ODkxMjk2Mzg2NzE5LCJsZWZ0Ijo5LjUyMzgwMzcxMDkzNzUsIndpZHRoIjozOS42NzA2NjU3NDA5NjY4LCJoZWlnaHQiOjEuNzYyNTg1NzU5MTYyOTAyOH19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjY1LjE1NjgxNDU3NTE5NTMxLCJsZWZ0Ijo5LjUyMzgwMzcxMDkzNzUsIndpZHRoIjozOS42NjYzMDU1NDE5OTIxOSwiaGVpZ2h0IjoxLjc2MjU4NTc1OTE2MjkwMjh9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo2Ni43MjQ3MTYxODY1MjM0NCwibGVmdCI6OS41MjM4MDM3MTA5Mzc1LCJ3aWR0aCI6MzkuMjg1OTgwMjI0NjA5Mzc1LCJoZWlnaHQiOjEuNzYyNTgxODI1MjU2MzQ3N319LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjY4LjI5MjYxNzc5Nzg1MTU2LCJsZWZ0Ijo5LjUyMzgwMzcxMDkzNzUsIndpZHRoIjozOS42NjY4MDkwODIwMzEyNSwiaGVpZ2h0IjoxLjc2MjU4MTgyNTI1NjM0Nzd9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo2OS44NjA1MjcwMzg1NzQyMiwibGVmdCI6OS41MjM4MDM3MTA5Mzc1LCJ3aWR0aCI6MzkuMjg1OTgwMjI0NjA5Mzc1LCJoZWlnaHQiOjEuNzYyNTgxODI1MjU2MzQ3N319LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjcxLjQyODQyMTAyMDUwNzgxLCJsZWZ0Ijo5LjUyMzgwMzcxMDkzNzUsIndpZHRoIjozOS42NjkzMjI5Njc1MjkzLCJoZWlnaHQiOjEuNzYyNTgxODI1MjU2MzQ3N319LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjcyLjk5NjMyMjYzMTgzNTk0LCJsZWZ0Ijo5LjUyMzgwMzcxMDkzNzUsIndpZHRoIjozOS42NjY0ODQ4MzI3NjM2NywiaGVpZ2h0IjoxLjc2MjU4MTgyNTI1NjM0Nzd9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo3NC41NjQyMjQyNDMxNjQwNiwibGVmdCI6OS41MjM4MDM3MTA5Mzc1LCJ3aWR0aCI6MTMuOTg1OTQ5NTE2Mjk2Mzg3LCJoZWlnaHQiOjEuNzYyNTgxODI1MjU2MzQ3N319LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjc5LjI1MzkzNjc2NzU3ODEyLCJsZWZ0IjoxOS4xNjQ2OTAwMTc3MDAxOTUsIndpZHRoIjoyMC4wMDI5OTQ1MzczNTM1MTYsImhlaWdodCI6MS42NzQ0NTI2NjI0Njc5NTY1fX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6ODIuMzI1MTM0Mjc3MzQzNzUsImxlZnQiOjkuNTIzNDQ5ODk3NzY2MTEzLCJ3aWR0aCI6MzkuNjY3NDgwNDY4NzUsImhlaWdodCI6OS42MDIwOTE3ODkyNDU2MDV9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo5LjI4NzAzNDk4ODQwMzMyLCJsZWZ0Ijo1MS4xODk4Mzg0MDk0MjM4Mywid2lkdGgiOjM5LjY2NjYxODM0NzE2Nzk3LCJoZWlnaHQiOjEuNzYyNTgyNzc4OTMwNjY0fX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6MTAuODU0OTM2NTk5NzMxNDQ1LCJsZWZ0Ijo1MS4xODk4Mzg0MDk0MjM4Mywid2lkdGgiOjM5LjY2MzgxODM1OTM3NSwiaGVpZ2h0IjoxLjc2MjU4Mjc3ODkzMDY2NH19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEyIn19LHsib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjMwLCJzdWJ0eXBlIjp7fSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQ2hlbiIsImZhbWlseSI6IlhpZSJ9fV0sImlkcyI6eyJpc3NuIjpbIjE2NjQtNDYyWCJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMzM4OS9mcGxzLjIwMjUuMTY3NTY1NyIsInBtaWQiOiI0MTAzNjM5MyIsIm9wZW5hbGV4IjoiVzQ0MTQyNDgzMDcifSwidGl0bGUiOiJUaGUgcm9sZSBvZiBtb2Rlcm4gYWdyaWN1bHR1cmFsIHRlY2hub2xvZ2llcyBpbiBpbXByb3ZpbmcgYWdyaWN1bHR1cmFsIHByb2R1Y3Rpdml0eSBhbmQgbGFuZCB1c2UgZWZmaWNpZW5jeSIsImFic3RyYWN0IjoiTW9kZXJuIGFncmljdWx0dXJhbCB0ZWNobm9sb2dpZXMgYXJlIGNydWNpYWwgZm9yIGFkZHJlc3NpbmcgZ2xvYmFsIGZvb2Qgc2VjdXJpdHkgYW5kIGVudmlyb25tZW50YWwgc3VzdGFpbmFiaWxpdHkgY2hhbGxlbmdlcyBhbWlkc3QgYSBncm93aW5nIHBvcHVsYXRpb24gYW5kIGNsaW1hdGUgY2hhbmdlLiBUaGVzZSBpbm5vdmF0aW9ucywgaW5jbHVkaW5nIHByZWNpc2lvbiBhZ3JpY3VsdHVyZSwgYmlvdGVjaG5vbG9neSwgc21hcnQgaXJyaWdhdGlvbiwgYXV0b21hdGlvbiwgdmVydGljYWwgZmFybWluZywgYW5kIGFydGlmaWNpYWwgaW50ZWxsaWdlbmNlIChBSSksIHNpZ25pZmljYW50bHkgZW5oYW5jZSBwcm9kdWN0aXZpdHkgYW5kIGxhbmQgdXNlIGVmZmljaWVuY3kuIFByZWNpc2lvbiBhZ3JpY3VsdHVyZSwgdXRpbGl6aW5nIEdQUywgZHJvbmVzLCBhbmQgSW9ULCBpbXByb3ZlcyB5aWVsZHMgYnkgMjDigJMzMCUgYW5kIGN1dHMgaW5wdXQgd2FzdGUgYnkgNDDigJM2MCUuIEJpb3RlY2hub2xvZ3ksIHdpdGggQ1JJU1BSIGFuZCBHTU9zLCBkZWxpdmVycyBkcm91Z2h0IGFuZCBwZXN0LXJlc2lzdGFudCBjcm9wcywgc3RhYmlsaXppbmcgeWllbGRzLCBhcyBzZWVuIHdpdGggQnQgY290dG9uIHJlZHVjaW5nIHBlc3RpY2lkZSB1c2UgYnkgNTAlIGluIEluZGlhLiBTbWFydCBpcnJpZ2F0aW9uIGJvb3N0cyB3YXRlciBlZmZpY2llbmN5IGJ5IDQw4oCTNjAlLCB3aGlsZSBhdXRvbWF0aW9uIGFuZCByb2JvdGljcyBtaXRpZ2F0ZSBsYWJvciBzaG9ydGFnZXMgYW5kIHJlZHVjZSBjb3N0cyBieSAyNSUuIFZlcnRpY2FsIGZhcm1pbmcgaW5jcmVhc2VzIHlpZWxkcyAxMOKAkzIwIHRpbWVzIHdpdGggOTUlIGxlc3MgbGFuZCBhbmQgd2F0ZXIsIHN1cHBvcnRpbmcgdXJiYW4gZm9vZCBzZWN1cml0eS4gQUkgYW5hbHl0aWNzIGVuaGFuY2UgZGVjaXNpb24tbWFraW5nIHdpdGggb3ZlciA5MCUgYWNjdXJhY3kgaW4gZm9yZWNhc3RpbmcgYW5kIHJlc291cmNlIGFsbG9jYXRpb24uIERlc3BpdGUgdGhlc2UgYmVuZWZpdHMsIGhpZ2ggY29zdHMsIHRlY2hub2xvZ2ljYWwgaWxsaXRlcmFjeSwgYW5kIHJlZ3VsYXRvcnkgaXNzdWVzIGhpbmRlciBhZG9wdGlvbiwgZXNwZWNpYWxseSBhbW9uZyBzbWFsbGhvbGRlcnMuIFBvbGljeSBzdXBwb3J0LCBwdWJsaWMtcHJpdmF0ZSBwYXJ0bmVyc2hpcHMsIGFuZCB0cmFpbmluZyBhcmUgdml0YWwgZm9yIGJyb2FkZXIgdGVjaG5vbG9neSBhY2Nlc3MgYW5kIGZhaXIgYmVuZWZpdHMuIEludGVncmF0aW5nIHJlbmV3YWJsZSBlbmVyZ3kgYW5kIGNpcmN1bGFyIGVjb25vbXkgcHJpbmNpcGxlcyBpbnRvIGFnZ3JvdGVjaCBwcmVzZW50cyBhIHBhdGggdG8gc3VzdGFpbmFiaWxpdHkuIFRoaXMgcmV2aWV3IGhpZ2hsaWdodHMgdGhlIHRyYW5zZm9ybWF0aXZlIHBvdGVudGlhbCBvZiBtb2Rlcm4gdGVjaG5vbG9naWVzIGZvciBzdXN0YWluYWJsZSBpbnRlbnNpZmljYXRpb24sIGluY3JlYXNpbmcgcHJvZHVjdGl2aXR5IHdpdGhvdXQgZXhwYW5kaW5nIGZhcm1sYW5kLCB3aGlsZSBsZXNzZW5pbmcgZW52aXJvbm1lbnRhbCBpbXBhY3RzLiBJdCB1bmRlcnNjb3JlcyB0aGUgbmVlZCBmb3IgY29vcmRpbmF0ZWQgZWZmb3J0cyB0byBvdmVyY29tZSBhZG9wdGlvbiBjaGFsbGVuZ2VzIGFuZCBoYXJuZXNzIHRoZXNlIGlubm92YXRpb25zIGZvciBnbG9iYWwgZm9vZCBzZWN1cml0eSBhbmQgY2xpbWF0ZSByZXNpbGllbmNlLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzg5L2ZwbHMuMjAyNS4xNjc1NjU3IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5mcm9udGllcnNpbi5vcmcvam91cm5hbHMvcGxhbnQtc2NpZW5jZS9hcnRpY2xlcy8xMC4zMzg5L2ZwbHMuMjAyNS4xNjc1NjU3L3BkZiIsImlzc3VlZERhdGUiOiIyMDI1LTA5LTE2IiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiRnJvbnRpZXJzIGluIFBsYW50IFNjaWVuY2UiLCJ2b2x1bWUiOjE2LCJpbXBhY3RGYWN0b3IiOiI3LjAwIn0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkZyb250aWVycyBNZWRpYSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6ODgsImNpdGVkQnlDb3VudCI6MH19LCJxdW90ZSI6IlN5bnRoZXRpYyBiaW9sb2d5IGlzIGFsc28gY29udHJpYnV0aW5nLCB3aXRoIG1pY3JvYmlhbCBjb25zb3J0aWEgZW5naW5lZXJlZCB0byBlbmhhbmNlIHNvaWwgaGVhbHRoIGFuZCBjcm9wIHJlc2lsaWVuY2UgKExhd3NvbiBldCBhbC4sIDIwMTkpLiBEZXNwaXRlIHRoZXNlIGFkdmFuY2VzLCBlcXVpdGFibGUgYWNjZXNzIHJlbWFpbnMgYSBjaGFsbGVuZ2UsIGFzIHNtYWxsaG9sZGVyIGZhcm1lcnMgaW4gZGV2ZWxvcGluZyByZWdpb25zIG9mdGVuIGxhY2sgYWNjZXNzIHRvIHBhdGVudGVkIHRlY2hub2xvZ2llcyAoVG9lbm5pZXNzZW4gZXQgYWwuLCAyMDA4KS4gSW5pdGlhdGl2ZXMgbGlrZSB0aGUgT3BlbkNSSVNQUiBwcm9qZWN0IGFpbSB0byBkZW1vY3JhdGl6ZSBnZW5lLWVkaXRpbmcgdG9vbHMgKENvaGVuLCAyMDIzKSwgd2hpbGUgcHVibGljLXNlY3RvciBwYXJ0bmVyc2hpcHMgKGUuZy4sIENJTU1ZVOKAmXMgZHJvdWdodC10b2xlcmFudCBtYWl6ZSkgZW5zdXJlIGFmZm9yZGFiaWxpdHkuIEFzIGNsaW1hdGUgY2hhbmdlIGludGVuc2lmaWVzLCB0aGVzZSBiaW90ZWNobm9sb2dpZXMgd2lsbCBiZSBpbmRpc3BlbnNhYmxlIGZvciBhY2hpZXZpbmcgc3VzdGFpbmFibGUgaW50ZW5zaWZpY2F0aW9uLCBidXQgdGhlaXIgc3VjY2VzcyBoaW5nZXMgb24gZ2xvYmFsIGNvbGxhYm9yYXRpb24sIGV0aGljYWwgZ292ZXJuYW5jZSwgYW5kIGluY2x1c2l2ZSBpbm5vdmF0aW9uLiAzLjMgU21hcnQgaXJyaWdhdGlvbiAmIHdhdGVyIG1hbmFnZW1lbnQgV2F0ZXIgc2NhcmNpdHkgcG9zZXMgb25lIG9mIHRoZSBtb3N0IHNpZ25pZmljYW50IGNoYWxsZW5nZXMgdG8gZ2xvYmFsIGFncmljdWx0dXJlLCB3aXRoIDcwJSBvZiBmcmVzaHdhdGVyIHdpdGhkcmF3YWxzIGFsbG9jYXRlZCB0byBpcnJpZ2F0aW9uLCBvZnRlbiBpbmVmZmljaWVudGx5IChGQU8sIDIwMjApLiBTbWFydCBpcnJpZ2F0aW9uIHN5c3RlbXMsIGludGVncmF0aW5nIElvVC1iYXNlZCBzZW5zb3JzIGFuZCBkcmlwIGlycmlnYXRpb24gdGVjaG5vbG9naWVzLCBhcmUgdHJhbnNmb3JtaW5nIHdhdGVyIG1hbmFnZW1lbnQgYnkgb3B0aW1pemluZyB1c2FnZSwgcmVkdWNpbmcgd2FzdGUsIGFuZCBlbmhhbmNpbmcgY3JvcCB5aWVsZHMuIFNvaWwgbW9pc3R1cmUgc2Vuc29ycywgc3VjaCBhcyBjYXBhY2l0aXZlIHByb2JlcyBhbmQgdGVuc2lvbWV0ZXJzLCBwcm92aWRlIHJlYWwtdGltZSBkYXRhIG9uIHdhdGVyIGF2YWlsYWJpbGl0eSBhdCB2YXJ5aW5nIHJvb3QgZGVwdGhzLCBlbmFibGluZyBwcmVjaXNlIGlycmlnYXRpb24gc2NoZWR1bGluZy4gVGhlc2Ugc2Vuc29ycyB0cmFuc21pdCBkYXRhIHZpYSBMb1JhV0FOIG9yIGNlbGx1bGFyIG5ldHdvcmtzIHRvIGNsb3VkIHBsYXRmb3Jtcywgd2hlcmUgQUkgYWxnb3JpdGhtcyBhbmFseXplIHRyZW5kcyBhbmQgYXV0b21hdGUgaXJyaWdhdGlvbiBkZWNpc2lvbnMgKEdvYXAgZXQgYWwuLCAyMDE4KS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo1Ny4xNzU2NDM5MjA4OTg0NCwibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ5MzE3NTUwNjU5MTgsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NTguNjY0MjM0MTYxMzc2OTUsImxlZnQiOjguMzMxNDQyODMyOTQ2Nzc3LCJ3aWR0aCI6NDAuNDkzOTUzNzA0ODMzOTg0LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjYwLjE0NjA4MDAxNzA4OTg0NCwibGVmdCI6OC4zMzE0NDE4NzkyNzI0NjEsIndpZHRoIjo0MC40OTc1ODE0ODE5MzM1OTQsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NjEuNjM0Njc3ODg2OTYyODksImxlZnQiOjguMzMxNDQxODc5MjcyNDYxLCJ3aWR0aCI6NDAuNDk1OTA2ODI5ODMzOTg0LCJoZWlnaHQiOjEuMTgwNTE2OTU4MjM2Njk0M319LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjYzLjExNjUyNzU1NzM3MzA1LCJsZWZ0Ijo4LjMzMTQ0MjgzMjk0Njc3Nywid2lkdGgiOjQwLjQ5NzU3NzY2NzIzNjMzLCJoZWlnaHQiOjEuMTgwNTE2OTU4MjM2Njk0M319LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjY0LjYwNTExNzc5Nzg1MTU2LCJsZWZ0Ijo4LjMzMTQzOTk3MTkyMzgyOCwid2lkdGgiOjQwLjQ4ODM0NjA5OTg1MzUxNiwiaGVpZ2h0IjoxLjE4MDUxNjk1ODIzNjY5NDN9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo2Ni4wODY5Njc0NjgyNjE3MiwibGVmdCI6OC4zMzE0NDE4NzkyNzI0NjEsIndpZHRoIjo0MC40ODMyMDc3MDI2MzY3MiwiaGVpZ2h0IjoxLjE4MDUxNjk1ODIzNjY5NDN9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo2Ny41NzU1NTM4OTQwNDI5NywibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ4MzA1MTMwMDA0ODgzLCJoZWlnaHQiOjEuMTgwNTE2OTU4MjM2Njk0M319LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjY5LjA1NzQwMzU2NDQ1MzEyLCJsZWZ0Ijo4LjMzMTU5MTYwNjE0MDEzNywid2lkdGgiOjQwLjQ4MTg2MTExNDUwMTk1LCJoZWlnaHQiOjEuMTgwNTE2OTU4MjM2Njk0M319LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjcwLjU0NTk5NzYxOTYyODksImxlZnQiOjguMzMxNTg5Njk4NzkxNTA0LCJ3aWR0aCI6NDAuNDg2MDgzOTg0Mzc1LCJoZWlnaHQiOjEuMTgwNTE2OTU4MjM2Njk0M319LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjcyLjAyNzg0NzI5MDAzOTA2LCJsZWZ0Ijo4LjMzMTQzOTAxODI0OTUxMiwid2lkdGgiOjM4LjIxMTg3MjEwMDgzMDA4LCJoZWlnaHQiOjEuMTgwNTE2OTU4MjM2Njk0M319LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjc2LjEzOTU0MTYyNTk3NjU2LCJsZWZ0Ijo4LjMzMTc0NzA1NTA1MzcxMSwid2lkdGgiOjM4LjQ1NjgyMTQ0MTY1MDM5LCJoZWlnaHQiOjEuNTU4MzMyMjA0ODE4NzI1Nn19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjc5LjQ1ODYyNTc5MzQ1NzAzLCJsZWZ0Ijo4LjMzMTU5OTIzNTUzNDY2OCwid2lkdGgiOjQwLjUwOTY1ODgxMzQ3NjU2LCJoZWlnaHQiOjExLjU3MzY4NjU5OTczMTQ0NX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjkuNjU1MTM2MTA4Mzk4NDM4LCJsZWZ0Ijo1MS4yMDkyMTcwNzE1MzMyLCJ3aWR0aCI6NDAuNDg1NjQ1Mjk0MTg5NDUsImhlaWdodCI6MS4xODA1MTk1ODA4NDEwNjQ1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6MTEuMTM2OTkxNTAwODU0NDkyLCJsZWZ0Ijo1MS4yMDkyMTcwNzE1MzMyLCJ3aWR0aCI6NDAuNDgzMTk2MjU4NTQ0OTIsImhlaWdodCI6MS4xODA1MTk1ODA4NDEwNjQ1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6MTIuNjI1NTgzNjQ4NjgxNjQsImxlZnQiOjUxLjIwOTIxNzA3MTUzMzIsIndpZHRoIjo0MC40OTA3NTY5ODg1MjUzOSwiaGVpZ2h0IjoxLjE4MDUxOTU4MDg0MTA2NDV9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI2In19XQ%3D%3D.

### Climate Change Adaptation and Resilience

Escalating climate variability demands adaptive strategies like drought-tolerant varieties and regulated deficit irrigation, which improve yield under stress but require precise timing to avoid losses (Chai et al., 2015; Chami & Moujabber, 2016)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkRhbmllbCBFbCIsImZhbWlseSI6IkNoYW1pIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1hcm91biBFbCIsImZhbWlseSI6Ik1vdWphYmJlciJ9fV0sImlkcyI6eyJpc3NuIjpbIjAwMzgtMjM1MyIsIjE5OTYtNzQ4OSJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMTcxNTkvc2Fqcy4yMDE2LzIwMTUwNDU3IiwibWFnIjoiMjUyMzY3OTY5OCIsIm9wZW5hbGV4IjoiVzI1MjM2Nzk2OTgifSwidGl0bGUiOiJEcm91Z2h0LCBjbGltYXRlIGNoYW5nZSBhbmQgc3VzdGFpbmFiaWxpdHkgb2Ygd2F0ZXIgaW4gYWdyaWN1bHR1cmU6IEEgcm9hZG1hcCB0b3dhcmRzIHRoZSBOV1JTMiIsImFic3RyYWN0IjoiVGhlIGZyZXF1ZW5jeSBhbmQgaW50ZW5zaXR5IG9mIGRyb3VnaHQsIGV4dHJlbWUgZXZlbnRzIGFuZCBoaWdoIHdpbmQgdmVsb2NpdGllcyBpbiBTb3V0aCBBZnJpY2EgYXJlIGV4cGVjdGVkIHRvIGluY3JlYXNlIGluIHRoZSBuZXh0IGNlbnR1cnkgYXMgYSByZXN1bHQgb2YgdGhlIGNoYW5naW5nIGNsaW1hdGUuIFRoZSBOYXRpb25hbCBXYXRlciBSZXNvdXJjZSBTdHJhdGVneSAyIChOV1JTMikgaGFzIHNldCBvdXQgdGhlIGdlbmVyYWwgYW5kIHN0cmF0ZWdpYyBkaXJlY3Rpb25zIGZvciB3YXRlciByZXNvdXJjZXMgbWFuYWdlbWVudCBpbiB0aGUgY291bnRyeSBmb3IgdGhlIG5leHQgMjAgeWVhcnMuIEhvd2V2ZXIsIHRoZSBzdHJhdGVneSBkb2VzIG5vdCBkcmF3IGEgZnJhbWV3b3JrIHRhaWxvcmVkIHNwZWNpZmljYWxseSBmb3IgYWdyaWN1bHR1cmFsIHVzZSwgd2l0aCBzcGVjaWZpYyBtZWFzdXJlcyBhbmQgZ29hbHMuIFRoZXJlZm9yZSwgdG8gcmVhY2ggc3VzdGFpbmFiaWxpdHkgb2Ygd2F0ZXIgaW4gYWdyaWN1bHR1cmUsIGZvdXIgbWFqb3Igc3RyYXRlZ2ljIGdvYWxzIGFyZSBzdWdnZXN0ZWQsIG9uIHdoaWNoIHJlc2VhcmNoIGluc3RpdHV0aW9ucyBjYW4gZm9jdXMgYW5kIHByb21vdGUgdGhyb3VnaCBnb29kIGdvdmVybmFuY2UuIFRoZSBzdHJhdGVneSBlbXBoYXNpc2VzOiAoMSkgY3JvcCByZXNlYXJjaCB0byBmaW5kIG5ldyBkcm91Z2h0LWFuZCBoZWF0LSB0b2xlcmFudCBhbmQgcmVzaXN0YW50IGJyZWVkcyBhbmQgdmFyaWV0aWVzOyAoMikgaW50ZW5zaWZpZWQgcmVzZWFyY2ggaW4gYWdyaWN1bHR1cmFsIHByYWN0aWNlczsgKDMpIGluY3JlYXNpbmcgdGhlIGVmZmljaWVuY3kgb2Ygd2F0ZXIgdXNlIHdpdGhpbiBhZ3JpY3VsdHVyZTsgYW5kICg0KSBpbnRlZ3JhdGluZyBhbGwgdGhlc2Ugc3RyYXRlZ2ljIGdvYWxzIHdpdGhpbiBhIHN1c3RhaW5hYmxlIHJlc2VhcmNoIGZyYW1ld29yay4gRmluYWxseSwgdGhlIHJlc2VhcmNoIGNhbGxzIGZvciByYXBpZCBhY3Rpb24gYW5kIGltcGxlbWVudGF0aW9uLiBTaWduaWZpY2FuY2U6IFRoZSBmcmFtZXdvcmsgaXMgcHJvcG9zZWQgZm9yIHN0YWtlaG9sZGVycyBhbmQgcG9saWN5bWFrZXJzIGluIGhpZ2hlciBlZHVjYXRpb24sIGFncmljdWx0dXJlIGFuZCByZXNvdXJjZXMgbWFuYWdlbWVudCBpbiBTb3V0aCBBZnJpY2EgZm9yIG5ldyByZXNlYXJjaCBob3Jpem9ucyBhdCBuYXRpb25hbCBsZXZlbCB0byBpbXByb3ZlIG92ZXJhbGwgYWdyaWN1bHR1cmFsIHN1c3RhaW5hYmlsaXR5IGJ5IDIwMzAgYXMgc3RpcHVsYXRlZCBieSB0aGUgTWlsbGVubml1bSBEZXZlbG9wbWVudCBHb2Fscy4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTcxNTkvc2Fqcy4yMDE2LzIwMTUwNDU3IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5zYWpzLmNvLnphL2FydGljbGUvZG93bmxvYWQvMzY2My80NzY5IiwiaXNzdWVkRGF0ZSI6IjIwMTYtMDktMjMiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA5IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJTb3V0aCBBZnJpY2FuIEpvdXJuYWwgb2YgU2NpZW5jZSIsInZvbHVtZSI6MTEyLCJpbXBhY3RGYWN0b3IiOiIxLjQyIn0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6NCwibGFzdFBhZ2UiOjR9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiQWNhZGVteSBvZiBTY2llbmNlIG9mIFNvdXRoIEFmcmljYSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MzksImNpdGVkQnlDb3VudCI6MjV9fSwicXVvdGUiOiJJbiB0aGUgbWVhbnRpbWUsIGNyb3AgYnJlZWRpbmcgKGVzcGVjaWFsbHkgdGhlICBjb252ZW50aW9uYWwgYnJlZWRpbmcgdXNpbmcgbWFya2VyLWFzc2lzdGVkIHNlbGVjdGlvbikgaXMgYW4gZXh0cmVtZWx5ICB1c2VmdWwgdGVjaG5pcXVlIGZvciBicmVlZGluZyBjb21wbGV4IHRyYWl0cyAoc3VjaCBhcyBkcm91Z2h0IHRvbGVyYW5jZSkgIGludG8gaW1wcm92ZWQgdmFyaWV0aWVzLjIyIFRoZXJlIGlzIGEgZGVhcnRoIG9mIHN0dWRpZXMgcmVwb3J0ZWQgaW4gdGhlIGxpdGVyYXR1cmUgdG8gZW5hYmxlIHF1YW50aS1maWNhdGlvbiBvZiB3YXRlciBjb25zdW1wdGlvbiBhbmQgc2F2aW5ncyBhcyBhIHJlc3VsdCBvZiBzdWNoIGltcHJvdmVkIHZhcmlldGllcy4gSG93ZXZlciwgYSBzdHVkeSBoYXMgc2hvd24gdGhhdCBkcm91Z2h0LXRvbGVyYW50IGFuZCBkcm91Z2h0LXJlc2lzdGFudCB0cmFpdHMgY291bGQgaW1wcm92ZSB0aGUgeWllbGQgb2YgbWFpemUgaW4gQWZyaWNhIGJ5IHVwIHRvIDMwJSAgdW5kZXIgZHJvdWdodCBjb25kaXRpb25zIHdoZW4gY29tcGFyZWQgd2l0aCBjb21tZXJjaWFsIHNlZWRzLjIzICBGdXJ0aGVybW9yZSwgR3VyIGFuZCBaYW1pcjI0IHN1Y2Nlc3NmdWxseSBvYnRhaW5lZCA1MCUgaGlnaGVyIHlpZWxkcyBvZiAgdG9tYXRvLCBldmVuIHVuZGVyIGRyb3VnaHQgY29uZGl0aW9ucywgYnkgYnJlZWRpbmcgY3VsdGl2YXRlZCB0b21hdG9lcyAgd2l0aCB3aWxkIHJlbGF0aXZlcy4gRGVyeW5nIGV0IGFsLjI1IGZvdW5kIOKAkyBvbiBhIGdsb2JhbCBzY2FsZSDigJMgdGhhdCBieSB0aGUgIDIwODBzLCBleHRyZW1lIGhlYXQgc3RyZXNzIGF0IGFudGhlc2lzIGNvdWxkOiAoMSkgZG91YmxlIGxvc3NlcyBvZiBtYWl6ZSAgeWllbGQsICgyKSByZWR1Y2UgcHJvamVjdGVkIGdhaW5zIGluIHNwcmluZyB3aGVhdCB5aWVsZCBieSBoYWxmLCBhbmQgKDMpICByZWR1Y2UgcHJvamVjdGVkIGdhaW5zIGluIHNveWJlYW4geWllbGQgYnkgYSBxdWFydGVyLiBBZ3JpY3VsdHVyYWwgcHJhY3RpY2VzIEJpb2RpdmVyc2l0eSBhbmQgaGVhbHRoeSBzb2lsIGFyZSBjZW50cmFsIHRvIGVjb2xvZ2ljYWwgYXBwcm9hY2hlcyBmb3IgbW9yZSBkcm91Z2h0LXJlc2lzdGFudCBhbmQgZHJvdWdodC10b2xlcmFudCBmYXJtaW5nIHByYWN0aWNlcywgd2hpY2ggaXMgIG1vcmUgcmVzaWxpZW50IHRvIGV4dHJlbWUgZXZlbnRzLjI2IFByYWN0aWNlcyBlbmNvdXJhZ2luZyBiaW9kaXZlcnNpdHkgIGFuZCBzb2lsIGhlYWx0aCB3b3VsZCBpbmNyZWFzZSB0aGUgYXZhaWxhYmxlIHdhdGVyIGZvciBjcm9wcyBhbmQgaW1wcm92ZSAgc3VzdGFpbmFiaWxpdHkgb2YgYWdyaWN1bHR1cmFsIHByb2R1Y3Rpb24uMjcgTWFueSBvZiB0aGVzZSBhZ3JpY3VsdHVyYWwgcHJhY3RpY2VzIGFyZSBhdmFpbGFibGUgYW5kIGFyZSB1c2VkIGluIGRpZmZlcmVudCBwbGFjZXMgaW4gdGhlIHdvcmxkIHRvIG1hbmFnZSBjcm9wcyBhbmQgc29pbHMuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDcuMzkwMTYzNDIxNjMwODYsImxlZnQiOjkuNTIzODAwODQ5OTE0NTUsIndpZHRoIjozOS4xNjYyOTAyODMyMDMxMjUsImhlaWdodCI6MS4yMTAyNzc5MTUwMDA5MTU1fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDguNjEyNjEzNjc3OTc4NTE2LCJsZWZ0Ijo5LjUyMzgwMDg0OTkxNDU1LCJ3aWR0aCI6MzkuMTYyODcyMzE0NDUzMTI1LCJoZWlnaHQiOjEuMjEwMjc3OTE1MDAwOTE1NX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQ5LjgzNTA1MjQ5MDIzNDM3NSwibGVmdCI6OS41MjM4MDA4NDk5MTQ1NSwid2lkdGgiOjM5LjE2NjE0NTMyNDcwNzAzLCJoZWlnaHQiOjEuMjEwMjc3OTE1MDAwOTE1NX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjUxLjA1NzQ5NTExNzE4NzUsImxlZnQiOjkuNTIzODAwODQ5OTE0NTUsIndpZHRoIjoxMy40MTU0MjYyNTQyNzI0NjEsImhlaWdodCI6MS4yMTAyNzc5MTUwMDA5MTU1fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTIuOTUzMzMwOTkzNjUyMzQ0LCJsZWZ0Ijo5LjUyMzgwMDg0OTkxNDU1LCJ3aWR0aCI6MzkuMTY4MjgxNTU1MTc1NzgsImhlaWdodCI6MTMuNDM0NzE1MjcwOTk2MDk0fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NjcuMTk1OTQ1NzM5NzQ2MSwibGVmdCI6OS41MjM4MDA4NDk5MTQ1NSwid2lkdGgiOjE3LjE1Njk2NTI1NTczNzMwNSwiaGVpZ2h0IjoxLjcxMDAxOTU4ODQ3MDQ1OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjY5LjE1NTM5NTUwNzgxMjUsImxlZnQiOjkuNTIzODAwODQ5OTE0NTUsIndpZHRoIjozOS4xNjczMDExNzc5Nzg1MTYsImhlaWdodCI6Ni4xMDAwNDk5NzI1MzQxOH19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjc1Ljk0MTAwMTg5MjA4OTg0LCJsZWZ0Ijo5LjUyMzgwMDg0OTkxNDU1LCJ3aWR0aCI6MzkuMTYyMTQ3NTIxOTcyNjU2LCJoZWlnaHQiOjEuMjEwMjc3OTE1MDAwOTE1NX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjc3LjE2MzQ1OTc3NzgzMjAzLCJsZWZ0Ijo5LjUyMzgwMDg0OTkxNDU1LCJ3aWR0aCI6MzkuMTYyNzE1OTExODY1MjM0LCJoZWlnaHQiOjEuMjEwMjc3OTE1MDAwOTE1NX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjUifX0seyJvcmlnaW4iOjQsIndvcmsiOnsidHlwZSI6MzAsInN1YnR5cGUiOnt9LCJjb250cmlidXRvcnMiOlt7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJRaWFuZyIsImZhbWlseSI6IkNoYWkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiWWFudGFpIiwiZmFtaWx5IjoiR2FuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkNhaSIsImZhbWlseSI6IlpoYW8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSHVpLWxpYW4iLCJmYW1pbHkiOiJYdSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJSZWFnYW4iLCJmYW1pbHkiOiJXYXNrb20ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiWWluaW5nIiwiZmFtaWx5IjoiTml1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkthZGFtYm90IEguIE0uIiwiZmFtaWx5IjoiU2lkZGlxdWUifX1dLCJpZHMiOnsiaXNzbiI6WyIxNzczLTAxNTUiLCIxNzc0LTA3NDYiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMDcvczEzNTkzLTAxNS0wMzM4LTYiLCJtYWciOiIyMjIzOTYxMjgzIiwib3BlbmFsZXgiOiJXMjIyMzk2MTI4MyJ9LCJ0aXRsZSI6IlJlZ3VsYXRlZCBkZWZpY2l0IGlycmlnYXRpb24gZm9yIGNyb3AgcHJvZHVjdGlvbiB1bmRlciBkcm91Z2h0IHN0cmVzcy4gQSByZXZpZXciLCJhYnN0cmFjdCI6IkFncmljdWx0dXJlIGNvbnN1bWVzIG1vcmUgdGhhbiB0d28gdGhpcmRzIG9mIHRoZSB0b3RhbCBmcmVzaHdhdGVyIG9mIHRoZSBwbGFuZXQuIFRoaXMgaXNzdWUgY2F1c2VzIHN1YnN0YW50aWFsIGNvbmZsaWN0IGluIGZyZXNod2F0ZXIgYWxsb2NhdGlvbiBiZXR3ZWVuIGFncmljdWx0dXJlIGFuZCBvdGhlciBlY29ub21pYyBzZWN0b3JzLiBSZWd1bGF0ZWQgZGVmaWNpdCBpcnJpZ2F0aW9uIChSREkpIGlzIGtleSB0ZWNobm9sb2d5IGJlY2F1c2UgaXQgaGVscHMgdG8gaW1wcm92ZSB3YXRlciB1c2UgZWZmaWNpZW5jeS4gTm9uZXRoZWxlc3MsIHRoZXJlIGlzIGEgbGFjayBvZiB1bmRlcnN0YW5kaW5nIG9mIHRoZSBtZWNoYW5pc21zIHdpdGggd2hpY2ggcGxhbnRzIHJlc3BvbmQgdG8gUkRJLiBJbiBwYXJ0aWN1bGFyLCBsaXR0bGUgaXMga25vd24gYWJvdXQgaG93IFJESSBtaWdodCBpbmNyZWFzZSBjcm9wIHByb2R1Y3Rpb24gd2hpbGUgcmVkdWNpbmcgdGhlIGFtb3VudCBvZiBpcnJpZ2F0aW9uIHdhdGVyIGluIHJlYWwtd29ybGQgYWdyaWN1bHR1cmUuIEluIHRoaXMgcmV2aWV3LCB3ZSBmb3VuZCB0aGF0IFJESSBpcyBsYXJnZWx5IGltcGxlbWVudGVkIHRocm91Z2ggdGhyZWUgYXBwcm9hY2hlczogKDEpIGdyb3d0aCBzdGFnZS1iYXNlZCBkZWZpY2l0IGlycmlnYXRpb24sICgyKSBwYXJ0aWFsIHJvb3Qtem9uZSBpcnJpZ2F0aW9uLCBhbmQgKDMpIHN1YnN1cmZhY2UgZHJpcHBlciBpcnJpZ2F0aW9uLiBBbW9uZyB0aGVzZSwgcGFydGlhbCByb290LXpvbmUgaXJyaWdhdGlvbiBpcyB0aGUgbW9zdCBwb3B1bGFyIGFuZCBlZmZlY3RpdmUgYmVjYXVzZSBtYW55IGZpZWxkIGNyb3BzIGFuZCBzb21lIHdvb2R5IGNyb3BzIGNhbiBzYXZlIGlycmlnYXRpb24gd2F0ZXIgdXAgdG8gMjAgdG8gMzAgJSB3aXRob3V0IG9yIHdpdGggYSBtaW5pbWFsIGltcGFjdCBvbiBjcm9wIHlpZWxkLiBJbXByb3ZlZCB3YXRlciB1c2UgZWZmaWNpZW5jeSB3aXRoIFJESSBpcyBtYWlubHkgZHVlIHRvIHRoZSBmb2xsb3dpbmc6ICgxKSBlbmhhbmNlZCBndWFyZCBjZWxsIHNpZ25hbCB0cmFuc2R1Y3Rpb24gbmV0d29yayB0aGF0IGRlY3JlYXNlcyB0cmFuc3BpcmF0aW9uIHdhdGVyIGxvc3MsICgyKSBvcHRpbWl6ZWQgc3RvbWF0YWwgY29udHJvbCB0aGF0IGltcHJvdmVzIHRoZSBwaG90b3N5bnRoZXNpcyB0byB0cmFuc3BpcmF0aW9uIHJhdGlvLCBhbmQgKDMpIGRlY3JlYXNlZCBldmFwb3JhdGl2ZSBzdXJmYWNlIGFyZWFzIHdpdGggcGFydGlhbCByb290LXpvbmUgaXJyaWdhdGlvbiB0aGF0IHJlZHVjZXMgc29pbCBldmFwb3JhdGlvbi4gVGhlIG1lY2hhbmlzbXMgaW52b2x2ZWQgaW4gdGhlIHBsYW50IHJlc3BvbnNlIHRvIFJESS1pbmR1Y2VkIHdhdGVyIHN0cmVzcyBpbmNsdWRlIHRoZSBtb3JwaG9sb2dpY2FsIHRyYWl0cywgZS5nLiwgaW5jcmVhc2VkIHJvb3QgdG8gc2hvb3QgcmF0aW8gYW5kIGltcHJvdmVkIG51dHJpZW50IHVwdGFrZSBhbmQgcmVjb3Zlcnk7IHBoeXNpb2xvZ2ljYWwgdHJhaXRzLCBlLmcuLCBzdG9tYXRhbCBjbG9zdXJlLCBkZWNyZWFzZWQgbGVhZiByZXNwaXJhdGlvbiwgYW5kIG1haW50YWluZWQgcGhvdG9zeW50aGVzaXM7IGFuZCBiaW9jaGVtaWNhbCB0cmFpdHMsIGUuZy4sIGluY3JlYXNlZCBzaWduYWxpbmcgbW9sZWN1bGVzIGFuZCBlbmhhbmNlZCBhbnRpb3hpZGF0aW9uIGVuenltYXRpYyBhY3Rpdml0eS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAwNy9zMTM1OTMtMDE1LTAzMzgtNiIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9saW5rLnNwcmluZ2VyLmNvbS9jb250ZW50L3BkZi8xMC4xMDA3L3MxMzU5My0wMTUtMDMzOC02LnBkZiIsImlzc3VlZERhdGUiOiIyMDE1LTEyLTE4IiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiQWdyb25vbXkgZm9yIFN1c3RhaW5hYmxlIERldmVsb3BtZW50IiwiaXNzdWUiOjEsInZvbHVtZSI6MzYsImltcGFjdEZhY3RvciI6IjkuNzYifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiU3ByaW5nZXIgU2NpZW5jZStCdXNpbmVzcyBNZWRpYSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MTQ4LCJjaXRlZEJ5Q291bnQiOjU1M319LCJxdW90ZSI6IkFkZGl0aW9uYWxseSwgdGhlIHJlYWxsb2NhdGlvbiBvZiB3YXRlciByZXNvdXJjZXMgYW5kIG1vZGlmaWNhdGlvbiBvZiBpcnJpZ2F0aW9uIHN5c3RlbXMgbWF5IGhhdmUgYSByb2xlIHRvIHBsYXkgZm9yIHRoZSBpbXByb3ZlbWVudCBvZiB3YXRlciB1c2UgaW4gYWdyaWN1bHR1cmUgKEhvbWF5b3VuZmFyIGV0IGFsLiAyMDE0KS4gNy4yIENoYWxsZW5nZXMgVGhlIGNlbnRyYWwgbm92ZWx0eSBvZiBSREkgaXMgdGhhdCBpdCBhbGxvd3MgcGxhbnRzIHRvIGdyb3cgdW5kZXIgbWlsZCB3YXRlciBzdHJlc3MgdG8gaW5kdWNlIHJvb3Qtc291cmNlZCBjaGVtaWNhbCBzaWctbmFscyBmcm9tIGRyeWluZyByb290cyB0byByZWFjaCB0aGUgc2hvb3Qgd2hlcmUgcGh5c2lvbG9naWNhbCBhbmQgYmlvY2hlbWljYWwgcHJvY2Vzc2VzIGNhbiBiZSByZWd1bGF0ZWQuIEhvd2V2ZXIsIHRoZSBwaHlzaW9sb2dpY2FsIGFuZCBiaW9jaGVtaWNhbCByZXNwb25zZXMgYXJlIGRpZmZpY3VsdCB0byBxdWFuLXRpZnksIGdpdmVuIHRoYXQgcGFydGlhbCByb290LXpvbmUgaXJyaWdhdGlvbiBpcyB1c3VhbGx5IGFwcGxpZWQgdG8gcGxhbnRzIGluIGFjY29yZGFuY2Ugd2l0aCBib3RoIHRlbXBvcmFsIGFuZCBzcGF0aWFsIGRyb3VnaHRzLiBJbiBwcmFjdGljZSwgYWRvcHRpb24gb2Ygd2F0ZXItc2F2aW5nIHRlY2huaXF1ZSByZXF1aXJlcyBzY2ktZW50aWZpYyB1bmRlcnN0YW5kaW5nIG9mIHRoZSBtZWNoYW5pc21zIGludm9sdmVkIGluIHRoZSBwaHlzaW9sb2dpY2FsIGFuZCBiaW9jaGVtaWNhbCBwcm9jZXNzZXMuIEFsc28sIHRoZSBvdXRjb21lIG9mIHRoZSB3YXRlciBzdHJlc3MtaW5kdWNlZCByZXNwb25zZXMgaXMgaW5mbHVlbmNlZCBieSBtYW55IGZhY3RvcnMsIHN1Y2ggYXMgdGhlIHN0YWdlcyBvZiBjcm9wIGdyb3d0aCB3aGVuIHRoZSBkZWZpY2l0IGlzIGFwcGxpZWQsIHRoZSBpbnRlbnNpdHkgYW5kIHNldmVyaXR5IG9mIGRlZmljaXQgaW1wb3NlZCwgdGhlIG1ldGhvZCB3aXRoIHdoaWNoIGRlZmljaXQgaXJyaWdhdGlvbiBpcyBhcHBsaWVkLCBjcm9wIHNwZWNpZXMgYW5kIGN1bHRpdmFycywgYW5kIHRoZWlyIHJlc3BlY3RpdmUgbm9uLWNyaXRpY2FsIGdyb3d0aCBzdGFnZXMgd2hlbiB0aGUgc3RhZ2UtYmFzZWQgZGVmaWNpdCBpcnJpZ2F0aW9uIG1heSBiZSBhcHBsaWVkLiBUaGVzZSBmYWN0b3JzIG5lZWQgdG8gYmUgaW52ZXN0aWdhdGVkIGF0IGEgc3lzdGVtIGxldmVsIHVuZGVyIHNwZWNpZmljIGdyb3dpbmcgY29uZGl0aW9ucy4gUkRJIGFuZCwgcGFydGljdWxhcmx5LCBhbHRlcm5hdGUgcGFydGlhbCByb290LXpvbmUgaXJyaWdhdGlvbiBhcmUgc3RpbGwgcmVsYXRpdmVseSBuZXcgdG8gbWFuYWdlIGZhcm0gbWFuYWdlcnMuIFRoaXMgdGVjaC1uaXF1ZSBpcyBub3QgZWFzeSB0byBpbXBsZW1lbnQgaW4gY3JvcCBzcGVjaWVzIHdpdGggZGVuc2UgcGxhbnQgcG9wdWxhdGlvbiBzdWNoIGFzIGNlcmVhbCBhbmQgb2lsc2VlZCBjcm9wcy4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEzLCJiYm94Ijp7InRvcCI6NjUuNTQxNDgxMDE4MDY2NCwibGVmdCI6NTEuNDMxODQ2NjE4NjUyMzQ0LCJ3aWR0aCI6MzkuOTk0MjgxNzY4Nzk4ODMsImhlaWdodCI6MS4zNjQyNjI3MDAwODA4NzE2fX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjY3LjExODY0NDcxNDM1NTQ3LCJsZWZ0Ijo1MS40MzE4NDY2MTg2NTIzNDQsIndpZHRoIjozOS45OTU5NDExNjIxMDkzNzUsImhlaWdodCI6MS4zNjQyNjI3MDAwODA4NzE2fX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjY4LjY4ODI0MDA1MTI2OTUzLCJsZWZ0Ijo1MS40MzE4NDY2MTg2NTIzNDQsIndpZHRoIjozOS45OTI1ODA0MTM4MTgzNiwiaGVpZ2h0IjoxLjM2NDI2MjcwMDA4MDg3MTZ9fSx7InBhZ2VOdW1iZXIiOjEzLCJiYm94Ijp7InRvcCI6NzAuMjY1Mzk2MTE4MTY0MDYsImxlZnQiOjUxLjQzMTg0NjYxODY1MjM0NCwid2lkdGgiOjE3Ljg5MDg5MjAyODgwODU5NCwiaGVpZ2h0IjoxLjM2NDI2MjcwMDA4MDg3MTZ9fSx7InBhZ2VOdW1iZXIiOjEzLCJiYm94Ijp7InRvcCI6NzMuNDA3MTA0NDkyMTg3NSwibGVmdCI6NTEuNDMxODQ2NjE4NjUyMzQ0LCJ3aWR0aCI6MTAuMTAzNTkxOTE4OTQ1MzEyLCJoZWlnaHQiOjEuMzc2ODU3NjM4MzU5MDY5OH19LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjo3Ni41NjY0NTk2NTU3NjE3MiwibGVmdCI6NTEuNDMxODQ2NjE4NjUyMzQ0LCJ3aWR0aCI6NDAuMDA3NjcxMzU2MjAxMTcsImhlaWdodCI6MTMuOTU4ODIwMzQzMDE3NTc4fX0seyJwYWdlTnVtYmVyIjoxNCwiYmJveCI6eyJ0b3AiOjcuMjg0ODU4NzAzNjEzMjgxLCJsZWZ0Ijo4LjU3MTQxODc2MjIwNzAzMSwid2lkdGgiOjQwLjAwNzY0MDgzODYyMzA1LCJoZWlnaHQiOjEuMzY0MjYzMTc2OTE4MDI5OH19LHsicGFnZU51bWJlciI6MTQsImJib3giOnsidG9wIjo4Ljg1NDQ1MzA4Njg1MzAyNywibGVmdCI6OC41NzE0MTg3NjIyMDcwMzEsIndpZHRoIjo0MC4wMTU5OTUwMjU2MzQ3NjYsImhlaWdodCI6MS4zNjQyNjM2NTM3NTUxODh9fSx7InBhZ2VOdW1iZXIiOjE0LCJiYm94Ijp7InRvcCI6MTAuNDMxNjExMDYxMDk2MTkxLCJsZWZ0Ijo4LjU3MTQxODc2MjIwNzAzMSwid2lkdGgiOjQwLjAwNzY0MDgzODYyMzA1LCJoZWlnaHQiOjEuMzY0MjYzNjUzNzU1MTg4fX0seyJwYWdlTnVtYmVyIjoxNCwiYmJveCI6eyJ0b3AiOjEyLjAwODc2MTQwNTk0NDgyNCwibGVmdCI6OC41NzE0MTg3NjIyMDcwMzEsIndpZHRoIjo0MC4wMDQzMDI5Nzg1MTU2MjUsImhlaWdodCI6MS4zNjQyNjM2NTM3NTUxODh9fSx7InBhZ2VOdW1iZXIiOjE0LCJiYm94Ijp7InRvcCI6MTMuNTc4MzU1Nzg5MTg0NTcsImxlZnQiOjguNTcxNDE4NzYyMjA3MDMxLCJ3aWR0aCI6MzkuOTk3NTk2NzQwNzIyNjU2LCJoZWlnaHQiOjEuMzY0MjYzNjUzNzU1MTg4fX0seyJwYWdlTnVtYmVyIjoxNCwiYmJveCI6eyJ0b3AiOjE1LjE1NTUxMzc2MzQyNzczNCwibGVmdCI6OC41NzE0MTg3NjIyMDcwMzEsIndpZHRoIjo0MC4wMTA5OTM5NTc1MTk1MywiaGVpZ2h0IjoxLjM2NDI2MzY1Mzc1NTE4OH19LHsicGFnZU51bWJlciI6MTQsImJib3giOnsidG9wIjoxNi43MzI2NjQxMDgyNzYzNjcsImxlZnQiOjguNTcxNDE4NzYyMjA3MDMxLCJ3aWR0aCI6NDAuMDEwOTkwMTQyODIyMjY2LCJoZWlnaHQiOjEuMzY0MjYyNzAwMDgwODcxNn19LHsicGFnZU51bWJlciI6MTQsImJib3giOnsidG9wIjoxOC4zMDIyNTk0NDUxOTA0MywibGVmdCI6OC41NzE0MTg3NjIyMDcwMzEsIndpZHRoIjo0MC4wMDc2MzcwMjM5MjU3OCwiaGVpZ2h0IjoxLjM2NDI2MjcwMDA4MDg3MTZ9fSx7InBhZ2VOdW1iZXIiOjE0LCJiYm94Ijp7InRvcCI6MTkuODc5NDE3NDE5NDMzNTk0LCJsZWZ0Ijo4LjU3MTQxODc2MjIwNzAzMSwid2lkdGgiOjEzLjA4OTMwNDkyNDAxMTIzLCJoZWlnaHQiOjEuMzY0MjYyNzAwMDgwODcxNn19LHsicGFnZU51bWJlciI6MTQsImJib3giOnsidG9wIjoyMS40NTY1Njc3NjQyODIyMjcsImxlZnQiOjEwLjQ3NTk4NzQzNDM4NzIwNywid2lkdGgiOjM4LjExMTQyNzMwNzEyODkwNiwiaGVpZ2h0IjoxLjM2NDI2MjcwMDA4MDg3MTZ9fSx7InBhZ2VOdW1iZXIiOjE0LCJiYm94Ijp7InRvcCI6MjMuMDI2MTYzMTAxMTk2MjksImxlZnQiOjguNTcxNDE4NzYyMjA3MDMxLCJ3aWR0aCI6MzkuOTkyNTg4MDQzMjEyODksImhlaWdodCI6MS4zNjQyNjI3MDAwODA4NzE2fX0seyJwYWdlTnVtYmVyIjoxNCwiYmJveCI6eyJ0b3AiOjI0LjYwMzMyMTA3NTQzOTQ1MywibGVmdCI6OC41NzE0MTg3NjIyMDcwMzEsIndpZHRoIjo0MC4wMDI2MTMwNjc2MjY5NSwiaGVpZ2h0IjoxLjM2NDI2MjcwMDA4MDg3MTZ9fSx7InBhZ2VOdW1iZXIiOjE0LCJiYm94Ijp7InRvcCI6MjYuMTgwNDY5NTEyOTM5NDUzLCJsZWZ0Ijo4LjU3MTQxODc2MjIwNzAzMSwid2lkdGgiOjM5Ljk5NTkyMjA4ODYyMzA1LCJoZWlnaHQiOjEuMzY0MjYyNzAwMDgwODcxNn19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjE0In19XQ%3D%3D. Breeding efforts, including marker-assisted selection, have shown up to 30-50% yield gains in maize and tomatoes under drought, yet lag behind rapid climate shifts (Chami & Moujabber, 2016)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkRhbmllbCBFbCIsImZhbWlseSI6IkNoYW1pIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1hcm91biBFbCIsImZhbWlseSI6Ik1vdWphYmJlciJ9fV0sImlkcyI6eyJpc3NuIjpbIjAwMzgtMjM1MyIsIjE5OTYtNzQ4OSJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMTcxNTkvc2Fqcy4yMDE2LzIwMTUwNDU3IiwibWFnIjoiMjUyMzY3OTY5OCIsIm9wZW5hbGV4IjoiVzI1MjM2Nzk2OTgifSwidGl0bGUiOiJEcm91Z2h0LCBjbGltYXRlIGNoYW5nZSBhbmQgc3VzdGFpbmFiaWxpdHkgb2Ygd2F0ZXIgaW4gYWdyaWN1bHR1cmU6IEEgcm9hZG1hcCB0b3dhcmRzIHRoZSBOV1JTMiIsImFic3RyYWN0IjoiVGhlIGZyZXF1ZW5jeSBhbmQgaW50ZW5zaXR5IG9mIGRyb3VnaHQsIGV4dHJlbWUgZXZlbnRzIGFuZCBoaWdoIHdpbmQgdmVsb2NpdGllcyBpbiBTb3V0aCBBZnJpY2EgYXJlIGV4cGVjdGVkIHRvIGluY3JlYXNlIGluIHRoZSBuZXh0IGNlbnR1cnkgYXMgYSByZXN1bHQgb2YgdGhlIGNoYW5naW5nIGNsaW1hdGUuIFRoZSBOYXRpb25hbCBXYXRlciBSZXNvdXJjZSBTdHJhdGVneSAyIChOV1JTMikgaGFzIHNldCBvdXQgdGhlIGdlbmVyYWwgYW5kIHN0cmF0ZWdpYyBkaXJlY3Rpb25zIGZvciB3YXRlciByZXNvdXJjZXMgbWFuYWdlbWVudCBpbiB0aGUgY291bnRyeSBmb3IgdGhlIG5leHQgMjAgeWVhcnMuIEhvd2V2ZXIsIHRoZSBzdHJhdGVneSBkb2VzIG5vdCBkcmF3IGEgZnJhbWV3b3JrIHRhaWxvcmVkIHNwZWNpZmljYWxseSBmb3IgYWdyaWN1bHR1cmFsIHVzZSwgd2l0aCBzcGVjaWZpYyBtZWFzdXJlcyBhbmQgZ29hbHMuIFRoZXJlZm9yZSwgdG8gcmVhY2ggc3VzdGFpbmFiaWxpdHkgb2Ygd2F0ZXIgaW4gYWdyaWN1bHR1cmUsIGZvdXIgbWFqb3Igc3RyYXRlZ2ljIGdvYWxzIGFyZSBzdWdnZXN0ZWQsIG9uIHdoaWNoIHJlc2VhcmNoIGluc3RpdHV0aW9ucyBjYW4gZm9jdXMgYW5kIHByb21vdGUgdGhyb3VnaCBnb29kIGdvdmVybmFuY2UuIFRoZSBzdHJhdGVneSBlbXBoYXNpc2VzOiAoMSkgY3JvcCByZXNlYXJjaCB0byBmaW5kIG5ldyBkcm91Z2h0LWFuZCBoZWF0LSB0b2xlcmFudCBhbmQgcmVzaXN0YW50IGJyZWVkcyBhbmQgdmFyaWV0aWVzOyAoMikgaW50ZW5zaWZpZWQgcmVzZWFyY2ggaW4gYWdyaWN1bHR1cmFsIHByYWN0aWNlczsgKDMpIGluY3JlYXNpbmcgdGhlIGVmZmljaWVuY3kgb2Ygd2F0ZXIgdXNlIHdpdGhpbiBhZ3JpY3VsdHVyZTsgYW5kICg0KSBpbnRlZ3JhdGluZyBhbGwgdGhlc2Ugc3RyYXRlZ2ljIGdvYWxzIHdpdGhpbiBhIHN1c3RhaW5hYmxlIHJlc2VhcmNoIGZyYW1ld29yay4gRmluYWxseSwgdGhlIHJlc2VhcmNoIGNhbGxzIGZvciByYXBpZCBhY3Rpb24gYW5kIGltcGxlbWVudGF0aW9uLiBTaWduaWZpY2FuY2U6IFRoZSBmcmFtZXdvcmsgaXMgcHJvcG9zZWQgZm9yIHN0YWtlaG9sZGVycyBhbmQgcG9saWN5bWFrZXJzIGluIGhpZ2hlciBlZHVjYXRpb24sIGFncmljdWx0dXJlIGFuZCByZXNvdXJjZXMgbWFuYWdlbWVudCBpbiBTb3V0aCBBZnJpY2EgZm9yIG5ldyByZXNlYXJjaCBob3Jpem9ucyBhdCBuYXRpb25hbCBsZXZlbCB0byBpbXByb3ZlIG92ZXJhbGwgYWdyaWN1bHR1cmFsIHN1c3RhaW5hYmlsaXR5IGJ5IDIwMzAgYXMgc3RpcHVsYXRlZCBieSB0aGUgTWlsbGVubml1bSBEZXZlbG9wbWVudCBHb2Fscy4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTcxNTkvc2Fqcy4yMDE2LzIwMTUwNDU3IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5zYWpzLmNvLnphL2FydGljbGUvZG93bmxvYWQvMzY2My80NzY5IiwiaXNzdWVkRGF0ZSI6IjIwMTYtMDktMjMiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA5IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJTb3V0aCBBZnJpY2FuIEpvdXJuYWwgb2YgU2NpZW5jZSIsInZvbHVtZSI6MTEyLCJpbXBhY3RGYWN0b3IiOiIxLjQyIn0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6NCwibGFzdFBhZ2UiOjR9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiQWNhZGVteSBvZiBTY2llbmNlIG9mIFNvdXRoIEFmcmljYSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MzksImNpdGVkQnlDb3VudCI6MjV9fSwicXVvdGUiOiJJbiB0aGUgbWVhbnRpbWUsIGNyb3AgYnJlZWRpbmcgKGVzcGVjaWFsbHkgdGhlICBjb252ZW50aW9uYWwgYnJlZWRpbmcgdXNpbmcgbWFya2VyLWFzc2lzdGVkIHNlbGVjdGlvbikgaXMgYW4gZXh0cmVtZWx5ICB1c2VmdWwgdGVjaG5pcXVlIGZvciBicmVlZGluZyBjb21wbGV4IHRyYWl0cyAoc3VjaCBhcyBkcm91Z2h0IHRvbGVyYW5jZSkgIGludG8gaW1wcm92ZWQgdmFyaWV0aWVzLjIyIFRoZXJlIGlzIGEgZGVhcnRoIG9mIHN0dWRpZXMgcmVwb3J0ZWQgaW4gdGhlIGxpdGVyYXR1cmUgdG8gZW5hYmxlIHF1YW50aS1maWNhdGlvbiBvZiB3YXRlciBjb25zdW1wdGlvbiBhbmQgc2F2aW5ncyBhcyBhIHJlc3VsdCBvZiBzdWNoIGltcHJvdmVkIHZhcmlldGllcy4gSG93ZXZlciwgYSBzdHVkeSBoYXMgc2hvd24gdGhhdCBkcm91Z2h0LXRvbGVyYW50IGFuZCBkcm91Z2h0LXJlc2lzdGFudCB0cmFpdHMgY291bGQgaW1wcm92ZSB0aGUgeWllbGQgb2YgbWFpemUgaW4gQWZyaWNhIGJ5IHVwIHRvIDMwJSAgdW5kZXIgZHJvdWdodCBjb25kaXRpb25zIHdoZW4gY29tcGFyZWQgd2l0aCBjb21tZXJjaWFsIHNlZWRzLjIzICBGdXJ0aGVybW9yZSwgR3VyIGFuZCBaYW1pcjI0IHN1Y2Nlc3NmdWxseSBvYnRhaW5lZCA1MCUgaGlnaGVyIHlpZWxkcyBvZiAgdG9tYXRvLCBldmVuIHVuZGVyIGRyb3VnaHQgY29uZGl0aW9ucywgYnkgYnJlZWRpbmcgY3VsdGl2YXRlZCB0b21hdG9lcyAgd2l0aCB3aWxkIHJlbGF0aXZlcy4gRGVyeW5nIGV0IGFsLjI1IGZvdW5kIOKAkyBvbiBhIGdsb2JhbCBzY2FsZSDigJMgdGhhdCBieSB0aGUgIDIwODBzLCBleHRyZW1lIGhlYXQgc3RyZXNzIGF0IGFudGhlc2lzIGNvdWxkOiAoMSkgZG91YmxlIGxvc3NlcyBvZiBtYWl6ZSAgeWllbGQsICgyKSByZWR1Y2UgcHJvamVjdGVkIGdhaW5zIGluIHNwcmluZyB3aGVhdCB5aWVsZCBieSBoYWxmLCBhbmQgKDMpICByZWR1Y2UgcHJvamVjdGVkIGdhaW5zIGluIHNveWJlYW4geWllbGQgYnkgYSBxdWFydGVyLiBBZ3JpY3VsdHVyYWwgcHJhY3RpY2VzIEJpb2RpdmVyc2l0eSBhbmQgaGVhbHRoeSBzb2lsIGFyZSBjZW50cmFsIHRvIGVjb2xvZ2ljYWwgYXBwcm9hY2hlcyBmb3IgbW9yZSBkcm91Z2h0LXJlc2lzdGFudCBhbmQgZHJvdWdodC10b2xlcmFudCBmYXJtaW5nIHByYWN0aWNlcywgd2hpY2ggaXMgIG1vcmUgcmVzaWxpZW50IHRvIGV4dHJlbWUgZXZlbnRzLjI2IFByYWN0aWNlcyBlbmNvdXJhZ2luZyBiaW9kaXZlcnNpdHkgIGFuZCBzb2lsIGhlYWx0aCB3b3VsZCBpbmNyZWFzZSB0aGUgYXZhaWxhYmxlIHdhdGVyIGZvciBjcm9wcyBhbmQgaW1wcm92ZSAgc3VzdGFpbmFiaWxpdHkgb2YgYWdyaWN1bHR1cmFsIHByb2R1Y3Rpb24uMjcgTWFueSBvZiB0aGVzZSBhZ3JpY3VsdHVyYWwgcHJhY3RpY2VzIGFyZSBhdmFpbGFibGUgYW5kIGFyZSB1c2VkIGluIGRpZmZlcmVudCBwbGFjZXMgaW4gdGhlIHdvcmxkIHRvIG1hbmFnZSBjcm9wcyBhbmQgc29pbHMuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDcuMzkwMTYzNDIxNjMwODYsImxlZnQiOjkuNTIzODAwODQ5OTE0NTUsIndpZHRoIjozOS4xNjYyOTAyODMyMDMxMjUsImhlaWdodCI6MS4yMTAyNzc5MTUwMDA5MTU1fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDguNjEyNjEzNjc3OTc4NTE2LCJsZWZ0Ijo5LjUyMzgwMDg0OTkxNDU1LCJ3aWR0aCI6MzkuMTYyODcyMzE0NDUzMTI1LCJoZWlnaHQiOjEuMjEwMjc3OTE1MDAwOTE1NX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQ5LjgzNTA1MjQ5MDIzNDM3NSwibGVmdCI6OS41MjM4MDA4NDk5MTQ1NSwid2lkdGgiOjM5LjE2NjE0NTMyNDcwNzAzLCJoZWlnaHQiOjEuMjEwMjc3OTE1MDAwOTE1NX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjUxLjA1NzQ5NTExNzE4NzUsImxlZnQiOjkuNTIzODAwODQ5OTE0NTUsIndpZHRoIjoxMy40MTU0MjYyNTQyNzI0NjEsImhlaWdodCI6MS4yMTAyNzc5MTUwMDA5MTU1fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTIuOTUzMzMwOTkzNjUyMzQ0LCJsZWZ0Ijo5LjUyMzgwMDg0OTkxNDU1LCJ3aWR0aCI6MzkuMTY4MjgxNTU1MTc1NzgsImhlaWdodCI6MTMuNDM0NzE1MjcwOTk2MDk0fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NjcuMTk1OTQ1NzM5NzQ2MSwibGVmdCI6OS41MjM4MDA4NDk5MTQ1NSwid2lkdGgiOjE3LjE1Njk2NTI1NTczNzMwNSwiaGVpZ2h0IjoxLjcxMDAxOTU4ODQ3MDQ1OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjY5LjE1NTM5NTUwNzgxMjUsImxlZnQiOjkuNTIzODAwODQ5OTE0NTUsIndpZHRoIjozOS4xNjczMDExNzc5Nzg1MTYsImhlaWdodCI6Ni4xMDAwNDk5NzI1MzQxOH19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjc1Ljk0MTAwMTg5MjA4OTg0LCJsZWZ0Ijo5LjUyMzgwMDg0OTkxNDU1LCJ3aWR0aCI6MzkuMTYyMTQ3NTIxOTcyNjU2LCJoZWlnaHQiOjEuMjEwMjc3OTE1MDAwOTE1NX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjc3LjE2MzQ1OTc3NzgzMjAzLCJsZWZ0Ijo5LjUyMzgwMDg0OTkxNDU1LCJ3aWR0aCI6MzkuMTYyNzE1OTExODY1MjM0LCJoZWlnaHQiOjEuMjEwMjc3OTE1MDAwOTE1NX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjUifX1d. Integrating these with resilient practices like biodiversity enhancement builds long-term farm resilience (Chami & Moujabber, 2016)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkRhbmllbCBFbCIsImZhbWlseSI6IkNoYW1pIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1hcm91biBFbCIsImZhbWlseSI6Ik1vdWphYmJlciJ9fV0sImlkcyI6eyJpc3NuIjpbIjAwMzgtMjM1MyIsIjE5OTYtNzQ4OSJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMTcxNTkvc2Fqcy4yMDE2LzIwMTUwNDU3IiwibWFnIjoiMjUyMzY3OTY5OCIsIm9wZW5hbGV4IjoiVzI1MjM2Nzk2OTgifSwidGl0bGUiOiJEcm91Z2h0LCBjbGltYXRlIGNoYW5nZSBhbmQgc3VzdGFpbmFiaWxpdHkgb2Ygd2F0ZXIgaW4gYWdyaWN1bHR1cmU6IEEgcm9hZG1hcCB0b3dhcmRzIHRoZSBOV1JTMiIsImFic3RyYWN0IjoiVGhlIGZyZXF1ZW5jeSBhbmQgaW50ZW5zaXR5IG9mIGRyb3VnaHQsIGV4dHJlbWUgZXZlbnRzIGFuZCBoaWdoIHdpbmQgdmVsb2NpdGllcyBpbiBTb3V0aCBBZnJpY2EgYXJlIGV4cGVjdGVkIHRvIGluY3JlYXNlIGluIHRoZSBuZXh0IGNlbnR1cnkgYXMgYSByZXN1bHQgb2YgdGhlIGNoYW5naW5nIGNsaW1hdGUuIFRoZSBOYXRpb25hbCBXYXRlciBSZXNvdXJjZSBTdHJhdGVneSAyIChOV1JTMikgaGFzIHNldCBvdXQgdGhlIGdlbmVyYWwgYW5kIHN0cmF0ZWdpYyBkaXJlY3Rpb25zIGZvciB3YXRlciByZXNvdXJjZXMgbWFuYWdlbWVudCBpbiB0aGUgY291bnRyeSBmb3IgdGhlIG5leHQgMjAgeWVhcnMuIEhvd2V2ZXIsIHRoZSBzdHJhdGVneSBkb2VzIG5vdCBkcmF3IGEgZnJhbWV3b3JrIHRhaWxvcmVkIHNwZWNpZmljYWxseSBmb3IgYWdyaWN1bHR1cmFsIHVzZSwgd2l0aCBzcGVjaWZpYyBtZWFzdXJlcyBhbmQgZ29hbHMuIFRoZXJlZm9yZSwgdG8gcmVhY2ggc3VzdGFpbmFiaWxpdHkgb2Ygd2F0ZXIgaW4gYWdyaWN1bHR1cmUsIGZvdXIgbWFqb3Igc3RyYXRlZ2ljIGdvYWxzIGFyZSBzdWdnZXN0ZWQsIG9uIHdoaWNoIHJlc2VhcmNoIGluc3RpdHV0aW9ucyBjYW4gZm9jdXMgYW5kIHByb21vdGUgdGhyb3VnaCBnb29kIGdvdmVybmFuY2UuIFRoZSBzdHJhdGVneSBlbXBoYXNpc2VzOiAoMSkgY3JvcCByZXNlYXJjaCB0byBmaW5kIG5ldyBkcm91Z2h0LWFuZCBoZWF0LSB0b2xlcmFudCBhbmQgcmVzaXN0YW50IGJyZWVkcyBhbmQgdmFyaWV0aWVzOyAoMikgaW50ZW5zaWZpZWQgcmVzZWFyY2ggaW4gYWdyaWN1bHR1cmFsIHByYWN0aWNlczsgKDMpIGluY3JlYXNpbmcgdGhlIGVmZmljaWVuY3kgb2Ygd2F0ZXIgdXNlIHdpdGhpbiBhZ3JpY3VsdHVyZTsgYW5kICg0KSBpbnRlZ3JhdGluZyBhbGwgdGhlc2Ugc3RyYXRlZ2ljIGdvYWxzIHdpdGhpbiBhIHN1c3RhaW5hYmxlIHJlc2VhcmNoIGZyYW1ld29yay4gRmluYWxseSwgdGhlIHJlc2VhcmNoIGNhbGxzIGZvciByYXBpZCBhY3Rpb24gYW5kIGltcGxlbWVudGF0aW9uLiBTaWduaWZpY2FuY2U6IFRoZSBmcmFtZXdvcmsgaXMgcHJvcG9zZWQgZm9yIHN0YWtlaG9sZGVycyBhbmQgcG9saWN5bWFrZXJzIGluIGhpZ2hlciBlZHVjYXRpb24sIGFncmljdWx0dXJlIGFuZCByZXNvdXJjZXMgbWFuYWdlbWVudCBpbiBTb3V0aCBBZnJpY2EgZm9yIG5ldyByZXNlYXJjaCBob3Jpem9ucyBhdCBuYXRpb25hbCBsZXZlbCB0byBpbXByb3ZlIG92ZXJhbGwgYWdyaWN1bHR1cmFsIHN1c3RhaW5hYmlsaXR5IGJ5IDIwMzAgYXMgc3RpcHVsYXRlZCBieSB0aGUgTWlsbGVubml1bSBEZXZlbG9wbWVudCBHb2Fscy4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTcxNTkvc2Fqcy4yMDE2LzIwMTUwNDU3IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5zYWpzLmNvLnphL2FydGljbGUvZG93bmxvYWQvMzY2My80NzY5IiwiaXNzdWVkRGF0ZSI6IjIwMTYtMDktMjMiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA5IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJTb3V0aCBBZnJpY2FuIEpvdXJuYWwgb2YgU2NpZW5jZSIsInZvbHVtZSI6MTEyLCJpbXBhY3RGYWN0b3IiOiIxLjQyIn0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6NCwibGFzdFBhZ2UiOjR9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiQWNhZGVteSBvZiBTY2llbmNlIG9mIFNvdXRoIEFmcmljYSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MzksImNpdGVkQnlDb3VudCI6MjV9fSwicXVvdGUiOiJJbiB0aGUgbWVhbnRpbWUsIGNyb3AgYnJlZWRpbmcgKGVzcGVjaWFsbHkgdGhlICBjb252ZW50aW9uYWwgYnJlZWRpbmcgdXNpbmcgbWFya2VyLWFzc2lzdGVkIHNlbGVjdGlvbikgaXMgYW4gZXh0cmVtZWx5ICB1c2VmdWwgdGVjaG5pcXVlIGZvciBicmVlZGluZyBjb21wbGV4IHRyYWl0cyAoc3VjaCBhcyBkcm91Z2h0IHRvbGVyYW5jZSkgIGludG8gaW1wcm92ZWQgdmFyaWV0aWVzLjIyIFRoZXJlIGlzIGEgZGVhcnRoIG9mIHN0dWRpZXMgcmVwb3J0ZWQgaW4gdGhlIGxpdGVyYXR1cmUgdG8gZW5hYmxlIHF1YW50aS1maWNhdGlvbiBvZiB3YXRlciBjb25zdW1wdGlvbiBhbmQgc2F2aW5ncyBhcyBhIHJlc3VsdCBvZiBzdWNoIGltcHJvdmVkIHZhcmlldGllcy4gSG93ZXZlciwgYSBzdHVkeSBoYXMgc2hvd24gdGhhdCBkcm91Z2h0LXRvbGVyYW50IGFuZCBkcm91Z2h0LXJlc2lzdGFudCB0cmFpdHMgY291bGQgaW1wcm92ZSB0aGUgeWllbGQgb2YgbWFpemUgaW4gQWZyaWNhIGJ5IHVwIHRvIDMwJSAgdW5kZXIgZHJvdWdodCBjb25kaXRpb25zIHdoZW4gY29tcGFyZWQgd2l0aCBjb21tZXJjaWFsIHNlZWRzLjIzICBGdXJ0aGVybW9yZSwgR3VyIGFuZCBaYW1pcjI0IHN1Y2Nlc3NmdWxseSBvYnRhaW5lZCA1MCUgaGlnaGVyIHlpZWxkcyBvZiAgdG9tYXRvLCBldmVuIHVuZGVyIGRyb3VnaHQgY29uZGl0aW9ucywgYnkgYnJlZWRpbmcgY3VsdGl2YXRlZCB0b21hdG9lcyAgd2l0aCB3aWxkIHJlbGF0aXZlcy4gRGVyeW5nIGV0IGFsLjI1IGZvdW5kIOKAkyBvbiBhIGdsb2JhbCBzY2FsZSDigJMgdGhhdCBieSB0aGUgIDIwODBzLCBleHRyZW1lIGhlYXQgc3RyZXNzIGF0IGFudGhlc2lzIGNvdWxkOiAoMSkgZG91YmxlIGxvc3NlcyBvZiBtYWl6ZSAgeWllbGQsICgyKSByZWR1Y2UgcHJvamVjdGVkIGdhaW5zIGluIHNwcmluZyB3aGVhdCB5aWVsZCBieSBoYWxmLCBhbmQgKDMpICByZWR1Y2UgcHJvamVjdGVkIGdhaW5zIGluIHNveWJlYW4geWllbGQgYnkgYSBxdWFydGVyLiBBZ3JpY3VsdHVyYWwgcHJhY3RpY2VzIEJpb2RpdmVyc2l0eSBhbmQgaGVhbHRoeSBzb2lsIGFyZSBjZW50cmFsIHRvIGVjb2xvZ2ljYWwgYXBwcm9hY2hlcyBmb3IgbW9yZSBkcm91Z2h0LXJlc2lzdGFudCBhbmQgZHJvdWdodC10b2xlcmFudCBmYXJtaW5nIHByYWN0aWNlcywgd2hpY2ggaXMgIG1vcmUgcmVzaWxpZW50IHRvIGV4dHJlbWUgZXZlbnRzLjI2IFByYWN0aWNlcyBlbmNvdXJhZ2luZyBiaW9kaXZlcnNpdHkgIGFuZCBzb2lsIGhlYWx0aCB3b3VsZCBpbmNyZWFzZSB0aGUgYXZhaWxhYmxlIHdhdGVyIGZvciBjcm9wcyBhbmQgaW1wcm92ZSAgc3VzdGFpbmFiaWxpdHkgb2YgYWdyaWN1bHR1cmFsIHByb2R1Y3Rpb24uMjcgTWFueSBvZiB0aGVzZSBhZ3JpY3VsdHVyYWwgcHJhY3RpY2VzIGFyZSBhdmFpbGFibGUgYW5kIGFyZSB1c2VkIGluIGRpZmZlcmVudCBwbGFjZXMgaW4gdGhlIHdvcmxkIHRvIG1hbmFnZSBjcm9wcyBhbmQgc29pbHMuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDcuMzkwMTYzNDIxNjMwODYsImxlZnQiOjkuNTIzODAwODQ5OTE0NTUsIndpZHRoIjozOS4xNjYyOTAyODMyMDMxMjUsImhlaWdodCI6MS4yMTAyNzc5MTUwMDA5MTU1fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDguNjEyNjEzNjc3OTc4NTE2LCJsZWZ0Ijo5LjUyMzgwMDg0OTkxNDU1LCJ3aWR0aCI6MzkuMTYyODcyMzE0NDUzMTI1LCJoZWlnaHQiOjEuMjEwMjc3OTE1MDAwOTE1NX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQ5LjgzNTA1MjQ5MDIzNDM3NSwibGVmdCI6OS41MjM4MDA4NDk5MTQ1NSwid2lkdGgiOjM5LjE2NjE0NTMyNDcwNzAzLCJoZWlnaHQiOjEuMjEwMjc3OTE1MDAwOTE1NX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjUxLjA1NzQ5NTExNzE4NzUsImxlZnQiOjkuNTIzODAwODQ5OTE0NTUsIndpZHRoIjoxMy40MTU0MjYyNTQyNzI0NjEsImhlaWdodCI6MS4yMTAyNzc5MTUwMDA5MTU1fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTIuOTUzMzMwOTkzNjUyMzQ0LCJsZWZ0Ijo5LjUyMzgwMDg0OTkxNDU1LCJ3aWR0aCI6MzkuMTY4MjgxNTU1MTc1NzgsImhlaWdodCI6MTMuNDM0NzE1MjcwOTk2MDk0fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NjcuMTk1OTQ1NzM5NzQ2MSwibGVmdCI6OS41MjM4MDA4NDk5MTQ1NSwid2lkdGgiOjE3LjE1Njk2NTI1NTczNzMwNSwiaGVpZ2h0IjoxLjcxMDAxOTU4ODQ3MDQ1OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjY5LjE1NTM5NTUwNzgxMjUsImxlZnQiOjkuNTIzODAwODQ5OTE0NTUsIndpZHRoIjozOS4xNjczMDExNzc5Nzg1MTYsImhlaWdodCI6Ni4xMDAwNDk5NzI1MzQxOH19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjc1Ljk0MTAwMTg5MjA4OTg0LCJsZWZ0Ijo5LjUyMzgwMDg0OTkxNDU1LCJ3aWR0aCI6MzkuMTYyMTQ3NTIxOTcyNjU2LCJoZWlnaHQiOjEuMjEwMjc3OTE1MDAwOTE1NX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjc3LjE2MzQ1OTc3NzgzMjAzLCJsZWZ0Ijo5LjUyMzgwMDg0OTkxNDU1LCJ3aWR0aCI6MzkuMTYyNzE1OTExODY1MjM0LCJoZWlnaHQiOjEuMjEwMjc3OTE1MDAwOTE1NX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjUifX1d.

### Integrated Water Resource Management

Integrated management necessitates reallocating water resources and modifying irrigation systems amid competing demands, with RDI and precision methods playing key roles but challenged by quantifying physiological responses (Borin, 2024; Chai et al., 2015)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1hdXJpemlvIiwiZmFtaWx5IjoiQm9yaW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyMDM4LTU2MjUiXSwiaXNibiI6W10sImRvaSI6IjEwLjM2MjUzL2lqYW0tMjMyMyIsIm9wZW5hbGV4IjoiVzQzOTEyMjg5NjIifSwidGl0bGUiOiJBIHdpc2UgaXJyaWdhdGlvbiB0byBjb250cmlidXRlIHRvIGludGVncmF0ZWQgd2F0ZXIgcmVzb3VyY2UgbWFuYWdlbWVudCIsImFic3RyYWN0IjoiSXJyaWdhdGVkIGFncmljdWx0dXJlIGFjY291bnRzIGZvciBhYm91dCAyMCUgb2YgY3VsdGl2YXRlZCBsYW5kcyB3b3JsZHdpZGUgd2hpbGUgY3VycmVudGx5IGdlbmVyYXRpbmcgYW4gZXN0aW1hdGVkIDQwJSBvZiBjcm9wIHByb2R1Y3Rpb24sIGFuZCB0d28tdGhpcmRzIG9mIGZ1dHVyZSBnYWlucyBpbiBjcm9wIHByb2R1Y3Rpb24gYXJlIGV4cGVjdGVkIHRvIGNvbWUgZnJvbSBpcnJpZ2F0ZWQgbGFuZHMuIFRoZXJlZm9yZSwgaXJyaWdhdGlvbiBpcyBzdHJhdGVnaWMgdG8gZW5zdXJlIGZvb2QgZm9yIHRoZSB3b3JsZOKAmXMgaW5jcmVhc2luZyBwb3B1bGF0aW9uIGFuZCBzbG93IGRvd24gdGhlIHBhY2Ugb2YgZGVmb3Jlc3RhdGlvbi4gSXJyaWdhdGVkIGFncmljdWx0dXJlIGFsc28gYWNjb3VudHMgZm9yIG1vcmUgdGhhbiA3MCUgb2YgZ2xvYmFsIHdhdGVyIHdpdGhkcmF3YWxzLCBhbmQgdGhlIHdheSBhZ3JpY3VsdHVyZSB1c2VzIGZyZXNod2F0ZXIgaXMgY3J1Y2lhbCBmb3IgZW5zdXJpbmcgYXZhaWxhYmlsaXR5IGZvciBvdGhlciBhY3Rpdml0aWVzIGFuZCBwcmVzZXJ2aW5nIHdhdGVyLXJlbGF0ZWQgZWNvc3lzdGVtcy4gVG8gZ2l2ZSBhbiBhZ3JpLWVudmlyb25tZW50YWwgaW50ZXJwcmV0YXRpb24gb2YgdGhlIHdhdGVyIGZsdXhlcyBpbnZvbHZlZCBpbiBpcnJpZ2F0aW9uLCB0aGUgdG90YWwgYW1vdW50IG9mIHdhdGVyIHdpdGhkcmF3biBmcm9tIGEgc291cmNlIGlzIGNhbGxlZCB3aGl0ZSB3YXRlciAoVykgYW5kIHN1YmRpdmlkZWQgYXMgZm9sbG93czogZ29sZCB3YXRlciAoRykgaXMgdGhlIGFtb3VudCB0aGF0IGlzIGFjdHVhbGx5IHVzZWQgYnkgdGhlIGNyb3BzLiBUaGlzIGZyYWN0aW9uIHJlcHJlc2VudHMgdGhlIHVsdGltYXRlIGdvYWwgb2YgaXJyaWdhdGlvbiwgd2hpY2ggaXMgdG8gaW5jcmVhc2UgY3JvcCBwcm9kdWN0aXZpdHkgYW5kIGZvb2QgYXZhaWxhYmlsaXR5LiBXZSBjYW4gYWxzbyBjb25zaWRlciB0aGlzIGFtb3VudCBhcyB0aGUgaXJyaWdhdGlvbiB3YXRlciBmb3IgY3JvcHMuIFRoZSBnb2xkIGNvbG9yIG1lYW5zIGFuIGluY29tZSBmb3IgdGhlIGZhcm07IGVtZXJhbGQgd2F0ZXIgKEUpIGlzIHRoZSBhbW91bnQgb2Ygd2F0ZXIgd2l0aGRyYXduIGZvciBpcnJpZ2F0aW9uIHRoYXQgZG9lcyBub3QgcmVhY2ggdGhlIGNyb3BzIGJ1dCBwcm92aWRlcyBlY29zeXN0ZW0gc2VydmljZXMsIGkuZS4sIHRoZSBiZW5lZml0cyB0aGF0IHBlb3BsZSBnZXQgZnJvbSBlY29zeXN0ZW1zLiBUaGlzIGFtb3VudCBvZiB3YXRlciBpcyB1c2VmdWwgZm9yIHRoZSBjb21tdW5pdHkgYW5kIGNhbiBiZSBpZGVudGlmaWVkIGFzIGlycmlnYXRpb24gb2YgdGhlIHRlcnJpdG9yeTsgcmVkIHdhdGVyIChSKSBpcyBsb3NzZXMsIGkuZS4sIHRoZSBmcmFjdGlvbiBvZiB3YXRlciB0aGF0IGhhcyB0byBiZSByZWR1Y2VkIGFzIG11Y2ggYXMgcG9zc2libGUgaWYgbm90IGVsaW1pbmF0ZWQuIEl0IGlzIGFuIGVjb25vbWljLCBzb2NpYWwgYW5kIGVudmlyb25tZW50YWwgY29zdDsgdGhlIHJlZCBjb2xvciBpcyBhc3NvY2lhdGVkIHdpdGggdGhlIHJlZCB0cmFmZmljIGxpZ2h0IHRoYXQgbWVhbnMgU1RPUCEgVGhlIHF1YW50aWZpY2F0aW9uIG9mIFcgaGFzIHRvIGJlIHByZWNlZGVkIGJ5IGEgY2FyZWZ1bCBldmFsdWF0aW9uIG9mIHRoZSBiZW5lZml0cyBhY2hpZXZhYmxlIHdpdGggaXJyaWdhdGlvbiBpbiByZWxhdGlvbiB0byB0aGUgZW52aXJvbm1lbnRhbCBjaGFyYWN0ZXJpc3RpY3Mgb2YgdGhlIGN1bHRpdmF0aW9uIGFyZWEuIFN0cmF0ZWdpYyBpc3N1ZXMgYXQgdGhpcyBzdGFnZSBhcmUgdGhlIGNob2ljZSBiZXR3ZWVuIGZ1bGwgb3IgZGVmaWNpdCBpcnJpZ2F0aW9uIGFuZCB0aGUgaW5kaXZpZHVhdGlvbiBvZiBvcHBvcnR1bml0aWVzIHRvIGV4cGxvcmF0aW9uIG9mIG9wcG9ydHVuaXRpZXMgdG8gaW5jcmVhc2UgdGhlIGF2YWlsYWJpbGl0eSBvZiB3YXRlciByZXNvdXJjZXMgYnkgdXNpbmcgbm9uLWNvbnZlbnRpb25hbCBzb3VyY2VzLiBUaGUgcXVhbnRpZmljYXRpb24gb2YgRyBhbmQgRSBmbHV4ZXMgaXMgbmVjZXNzYXJ5IHRvIHJlZHVjZSBsb3NzZXMsIHdoaWxlIHN0cmF0ZWdpZXMvdGVjaG5pcXVlcyB1c2VmdWwgdG8gcmVkdWNlIGlycmlnYXRpb24gbmVlZHMgYXQgdGhlIGZpZWxkIGxldmVsIGFyZSBrZXkgZmFjdG9ycyB0byBhbGxvY2F0ZSBpcnJpZ2F0aW9uIHdhdGVyIHdpdGhpbiB0aGUgZnJhbWV3b3JrIG9mIGFuIGludGVncmF0ZWQgYW5kIHN1c3RhaW5hYmxlIG1hbmFnZW1lbnQgb2Ygd2F0ZXIgYWltZWQgYXQgdHVybmluZyB1c2VyIGNvbmZsaWN0cyBpbnRvIHN5bmVyZ2llcy4gVGhpcyBpbXBsaWVzIG11bHRpcGxlIHN1YmplY3RzIGFuZCBhY3RvcnMgaW4gYSBtdWx0aS1kaXNjaXBsaW5hcnkgYXBwcm9hY2guIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjM2MjUzL2lqYW0tMjMyMyIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9yaXZpc3RlLmZ1cHJlc3MubmV0L2luZGV4LnBocC9JSkFtL2FydGljbGUvZG93bmxvYWQvMjMyMy8xNjIxIiwiaXNzdWVkRGF0ZSI6IjIwMjQtMDEtMjAiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTEwIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJJdGFsaWFuIEpvdXJuYWwgb2YgQWdyb21ldGVvcm9sb2d5IiwiaXNzdWUiOjIsImltcGFjdEZhY3RvciI6IjEuMDAifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjo1LCJsYXN0UGFnZSI6MTl9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRmlyZW56ZSBVbml2ZXJzaXR5IFByZXNzIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjozOSwiY2l0ZWRCeUNvdW50IjoyfX0sInF1b3RlIjoiUHJvbWlzaW5nIHJlc3VsdHMgYXJlICByZWxhdGVkIHRvIHRoZSBhdmFpbGFiaWxpdHkgb2YgZHJvdWdodC10b2xlcmFudCBtYWl6ZSB2YXJpLWV0aWVzIHRoYXQgeWllbGQgbW9yZSBpbiBkcm91Z2h0LXN0cmVzc2VkIGVudmlyb25tZW50cyAgd2l0aCBubyBwZW5hbHR5IGluIG5vbi1zdHJlc3NlZCBlbnZpcm9ubWVudHMgKEFkZWUgIGV0IGFsLiwgMjAxNiksIGFuZCByZXF1aXJlIGxlc3Mgd2F0ZXIgdG8gbWF4aW1pemUgZ3JhaW4gIHlpZWxkIGFzIGNvbXBhcmVkIHRvIHRoZSBjb252ZW50aW9uYWwgaHlicmlkIChNb3VuY2UgIGV0IGFsLiwgMjAxNSkuIFRoZXJlZm9yZSwgY2FyZWZ1bCBzZWxlY3Rpb24gb2YgaHlicmlkcyBjYW4gIGluY3JlYXNlIGNvcm4geWllbGQgYW5kIFdVRSB1bmRlciB3YXRlci1saW1pdGVkIGNvbi1kaXRpb25zIChIYW8gZXQgYWwuLCAyMDE1KS4gVGhlIGFkZXF1YXRlIGNob2ljZSBvZiBjcm9wICBhbmQgdmFyaWV0eSBoYXMgdG8gYmUgYWNjb21wYW5pZWQgYnkgYWxsIHRoZSBvdGhlciAgb3B0aW9ucyBwZXJtaXR0aW5nIHRvIGluY3JlYXNlIHRoZSBwcm9wb3J0aW9uIG9mIGV2YXBvLXRyYW5zcGlyYXRpb24gZmxvd2luZyBhcyBwcm9kdWN0aXZlIHRyYW5zcGlyYXRpb24gYXMgdG8gIG9idGFpbiDigJxtb3JlIGNyb3AgcGVyIGRyb3DigJ0uIEFkZXF1YXRlIHRpbWluZyBhbmQgc3BhYy1pbmcgb2Ygc293aW5nLCB3ZWVkIGFuZCBwZXN0IGNvbnRyb2wsIG11bGNoaW5nLCBhcmUgb25seSAgc29tZSBleGFtcGxlcyBvZiB0aGUgd2lkZSByYW5nZSBvZiB0aGUgb3Bwb3J0dW5pdGllcyAgdGhhdCBjYW4gYmUgYWRvcHRlZC4gNS4gQ09OQ0xVRElORyBSRU1BUktTIElycmlnYXRpb24gaXMgaW5kaXNwZW5zYWJsZSB0byBhY2hpZXZlIHRoZSBaZXJvIGh1bi1nZXIgdGFyZ2V0IG9mIHRoZSAyMDMwIFVOIEFnZW5kYSBiZWNhdXNlIGl0IGlzIGEgcG93ZXItZnVsIHRvb2wgZm9yIHN1c3RhaW5hYmxlIGludGVuc2lmaWNhdGlvbiwgYW5kIGFpbXMgdG8gcHJvLWR1Y2UgbW9yZSBwZXIgc3VyZmFjZSB1bml0LiBUYWNrbGluZyB0aGUgaXNzdWUgb2YgZW5zdXJpbmcgbW9yZSBudXRyaXRpb3VzIGZvb2QgIGZvciBhIGdyb3dpbmcgcG9wdWxhdGlvbiwgdGhlIGhpZ2hlciBwcm9kdWN0aXZpdHkgb2YgaXJyaWdhdGVkIGxhbmRzIGNhbiBzbG93IGRvd24gdGhlIHBhY2Ugb2YgZGVmb3Jlc3RhdGlvbiwgaGVuY2Ug4oCcbW9yZSBpcnJpZ2F0aW9uLCBsZXNzIGRlZm9yZXN0YXRpb27igJ0uICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NTEuMDQ1NjkyNDQzODQ3NjU2LCJsZWZ0Ijo5LjUyMzgwMzcxMDkzNzUsIndpZHRoIjozOS42NzI1MzExMjc5Mjk2OSwiaGVpZ2h0IjoxLjc2MjU4NTc1OTE2MjkwMjh9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo1Mi42MTM1OTc4Njk4NzMwNSwibGVmdCI6OS41MjM4MDM3MTA5Mzc1LCJ3aWR0aCI6MzkuMjg1OTgwMjI0NjA5Mzc1LCJoZWlnaHQiOjEuNzYyNTg1NzU5MTYyOTAyOH19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjU0LjE4MTQ5OTQ4MTIwMTE3LCJsZWZ0Ijo5LjUyMzgwMzcxMDkzNzUsIndpZHRoIjozOS42NjEyNTg2OTc1MDk3NjYsImhlaWdodCI6MS43NjI1ODU3NTkxNjI5MDI4fX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NTUuNzQ5Mzk3Mjc3ODMyMDMsImxlZnQiOjkuNTIzODAzNzEwOTM3NSwid2lkdGgiOjM5LjY2NzE0ODU5MDA4Nzg5LCJoZWlnaHQiOjEuNzYyNTg1NzU5MTYyOTAyOH19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjU3LjMxNzMwNjUxODU1NDY5LCJsZWZ0Ijo5LjUyMzgwMzcxMDkzNzUsIndpZHRoIjozOS42NzMzNTUxMDI1MzkwNiwiaGVpZ2h0IjoxLjc2MjU4NTc1OTE2MjkwMjh9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo1OC44ODUyMDQzMTUxODU1NSwibGVmdCI6OS41MjM4MDM3MTA5Mzc1LCJ3aWR0aCI6MzkuNjY3OTk5MjY3NTc4MTI1LCJoZWlnaHQiOjEuNzYyNTg1NzU5MTYyOTAyOH19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjYwLjQ1MzEwNTkyNjUxMzY3LCJsZWZ0Ijo5LjUyMzgwMzcxMDkzNzUsIndpZHRoIjozOS42NjQyOTEzODE4MzU5NCwiaGVpZ2h0IjoxLjc2MjU4NTc1OTE2MjkwMjh9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo2Mi4wMjEwMDM3MjMxNDQ1MywibGVmdCI6OS41MjM4MDM3MTA5Mzc1LCJ3aWR0aCI6MzkuMjg1OTgwMjI0NjA5Mzc1LCJoZWlnaHQiOjEuNzYyNTg1NzU5MTYyOTAyOH19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjYzLjU4ODkxMjk2Mzg2NzE5LCJsZWZ0Ijo5LjUyMzgwMzcxMDkzNzUsIndpZHRoIjozOS42NzA2NjU3NDA5NjY4LCJoZWlnaHQiOjEuNzYyNTg1NzU5MTYyOTAyOH19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjY1LjE1NjgxNDU3NTE5NTMxLCJsZWZ0Ijo5LjUyMzgwMzcxMDkzNzUsIndpZHRoIjozOS42NjYzMDU1NDE5OTIxOSwiaGVpZ2h0IjoxLjc2MjU4NTc1OTE2MjkwMjh9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo2Ni43MjQ3MTYxODY1MjM0NCwibGVmdCI6OS41MjM4MDM3MTA5Mzc1LCJ3aWR0aCI6MzkuMjg1OTgwMjI0NjA5Mzc1LCJoZWlnaHQiOjEuNzYyNTgxODI1MjU2MzQ3N319LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjY4LjI5MjYxNzc5Nzg1MTU2LCJsZWZ0Ijo5LjUyMzgwMzcxMDkzNzUsIndpZHRoIjozOS42NjY4MDkwODIwMzEyNSwiaGVpZ2h0IjoxLjc2MjU4MTgyNTI1NjM0Nzd9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo2OS44NjA1MjcwMzg1NzQyMiwibGVmdCI6OS41MjM4MDM3MTA5Mzc1LCJ3aWR0aCI6MzkuMjg1OTgwMjI0NjA5Mzc1LCJoZWlnaHQiOjEuNzYyNTgxODI1MjU2MzQ3N319LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjcxLjQyODQyMTAyMDUwNzgxLCJsZWZ0Ijo5LjUyMzgwMzcxMDkzNzUsIndpZHRoIjozOS42NjkzMjI5Njc1MjkzLCJoZWlnaHQiOjEuNzYyNTgxODI1MjU2MzQ3N319LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjcyLjk5NjMyMjYzMTgzNTk0LCJsZWZ0Ijo5LjUyMzgwMzcxMDkzNzUsIndpZHRoIjozOS42NjY0ODQ4MzI3NjM2NywiaGVpZ2h0IjoxLjc2MjU4MTgyNTI1NjM0Nzd9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo3NC41NjQyMjQyNDMxNjQwNiwibGVmdCI6OS41MjM4MDM3MTA5Mzc1LCJ3aWR0aCI6MTMuOTg1OTQ5NTE2Mjk2Mzg3LCJoZWlnaHQiOjEuNzYyNTgxODI1MjU2MzQ3N319LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjc5LjI1MzkzNjc2NzU3ODEyLCJsZWZ0IjoxOS4xNjQ2OTAwMTc3MDAxOTUsIndpZHRoIjoyMC4wMDI5OTQ1MzczNTM1MTYsImhlaWdodCI6MS42NzQ0NTI2NjI0Njc5NTY1fX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6ODIuMzI1MTM0Mjc3MzQzNzUsImxlZnQiOjkuNTIzNDQ5ODk3NzY2MTEzLCJ3aWR0aCI6MzkuNjY3NDgwNDY4NzUsImhlaWdodCI6OS42MDIwOTE3ODkyNDU2MDV9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo5LjI4NzAzNDk4ODQwMzMyLCJsZWZ0Ijo1MS4xODk4Mzg0MDk0MjM4Mywid2lkdGgiOjM5LjY2NjYxODM0NzE2Nzk3LCJoZWlnaHQiOjEuNzYyNTgyNzc4OTMwNjY0fX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6MTAuODU0OTM2NTk5NzMxNDQ1LCJsZWZ0Ijo1MS4xODk4Mzg0MDk0MjM4Mywid2lkdGgiOjM5LjY2MzgxODM1OTM3NSwiaGVpZ2h0IjoxLjc2MjU4Mjc3ODkzMDY2NH19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEyIn19LHsib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjMwLCJzdWJ0eXBlIjp7fSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUWlhbmciLCJmYW1pbHkiOiJDaGFpIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllhbnRhaSIsImZhbWlseSI6IkdhbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJDYWkiLCJmYW1pbHkiOiJaaGFvIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikh1aS1saWFuIiwiZmFtaWx5IjoiWHUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUmVhZ2FuIiwiZmFtaWx5IjoiV2Fza29tIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllpbmluZyIsImZhbWlseSI6Ik5pdSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLYWRhbWJvdCBILiBNLiIsImZhbWlseSI6IlNpZGRpcXVlIn19XSwiaWRzIjp7Imlzc24iOlsiMTc3My0wMTU1IiwiMTc3NC0wNzQ2Il0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDA3L3MxMzU5My0wMTUtMDMzOC02IiwibWFnIjoiMjIyMzk2MTI4MyIsIm9wZW5hbGV4IjoiVzIyMjM5NjEyODMifSwidGl0bGUiOiJSZWd1bGF0ZWQgZGVmaWNpdCBpcnJpZ2F0aW9uIGZvciBjcm9wIHByb2R1Y3Rpb24gdW5kZXIgZHJvdWdodCBzdHJlc3MuIEEgcmV2aWV3IiwiYWJzdHJhY3QiOiJBZ3JpY3VsdHVyZSBjb25zdW1lcyBtb3JlIHRoYW4gdHdvIHRoaXJkcyBvZiB0aGUgdG90YWwgZnJlc2h3YXRlciBvZiB0aGUgcGxhbmV0LiBUaGlzIGlzc3VlIGNhdXNlcyBzdWJzdGFudGlhbCBjb25mbGljdCBpbiBmcmVzaHdhdGVyIGFsbG9jYXRpb24gYmV0d2VlbiBhZ3JpY3VsdHVyZSBhbmQgb3RoZXIgZWNvbm9taWMgc2VjdG9ycy4gUmVndWxhdGVkIGRlZmljaXQgaXJyaWdhdGlvbiAoUkRJKSBpcyBrZXkgdGVjaG5vbG9neSBiZWNhdXNlIGl0IGhlbHBzIHRvIGltcHJvdmUgd2F0ZXIgdXNlIGVmZmljaWVuY3kuIE5vbmV0aGVsZXNzLCB0aGVyZSBpcyBhIGxhY2sgb2YgdW5kZXJzdGFuZGluZyBvZiB0aGUgbWVjaGFuaXNtcyB3aXRoIHdoaWNoIHBsYW50cyByZXNwb25kIHRvIFJESS4gSW4gcGFydGljdWxhciwgbGl0dGxlIGlzIGtub3duIGFib3V0IGhvdyBSREkgbWlnaHQgaW5jcmVhc2UgY3JvcCBwcm9kdWN0aW9uIHdoaWxlIHJlZHVjaW5nIHRoZSBhbW91bnQgb2YgaXJyaWdhdGlvbiB3YXRlciBpbiByZWFsLXdvcmxkIGFncmljdWx0dXJlLiBJbiB0aGlzIHJldmlldywgd2UgZm91bmQgdGhhdCBSREkgaXMgbGFyZ2VseSBpbXBsZW1lbnRlZCB0aHJvdWdoIHRocmVlIGFwcHJvYWNoZXM6ICgxKSBncm93dGggc3RhZ2UtYmFzZWQgZGVmaWNpdCBpcnJpZ2F0aW9uLCAoMikgcGFydGlhbCByb290LXpvbmUgaXJyaWdhdGlvbiwgYW5kICgzKSBzdWJzdXJmYWNlIGRyaXBwZXIgaXJyaWdhdGlvbi4gQW1vbmcgdGhlc2UsIHBhcnRpYWwgcm9vdC16b25lIGlycmlnYXRpb24gaXMgdGhlIG1vc3QgcG9wdWxhciBhbmQgZWZmZWN0aXZlIGJlY2F1c2UgbWFueSBmaWVsZCBjcm9wcyBhbmQgc29tZSB3b29keSBjcm9wcyBjYW4gc2F2ZSBpcnJpZ2F0aW9uIHdhdGVyIHVwIHRvIDIwIHRvIDMwICUgd2l0aG91dCBvciB3aXRoIGEgbWluaW1hbCBpbXBhY3Qgb24gY3JvcCB5aWVsZC4gSW1wcm92ZWQgd2F0ZXIgdXNlIGVmZmljaWVuY3kgd2l0aCBSREkgaXMgbWFpbmx5IGR1ZSB0byB0aGUgZm9sbG93aW5nOiAoMSkgZW5oYW5jZWQgZ3VhcmQgY2VsbCBzaWduYWwgdHJhbnNkdWN0aW9uIG5ldHdvcmsgdGhhdCBkZWNyZWFzZXMgdHJhbnNwaXJhdGlvbiB3YXRlciBsb3NzLCAoMikgb3B0aW1pemVkIHN0b21hdGFsIGNvbnRyb2wgdGhhdCBpbXByb3ZlcyB0aGUgcGhvdG9zeW50aGVzaXMgdG8gdHJhbnNwaXJhdGlvbiByYXRpbywgYW5kICgzKSBkZWNyZWFzZWQgZXZhcG9yYXRpdmUgc3VyZmFjZSBhcmVhcyB3aXRoIHBhcnRpYWwgcm9vdC16b25lIGlycmlnYXRpb24gdGhhdCByZWR1Y2VzIHNvaWwgZXZhcG9yYXRpb24uIFRoZSBtZWNoYW5pc21zIGludm9sdmVkIGluIHRoZSBwbGFudCByZXNwb25zZSB0byBSREktaW5kdWNlZCB3YXRlciBzdHJlc3MgaW5jbHVkZSB0aGUgbW9ycGhvbG9naWNhbCB0cmFpdHMsIGUuZy4sIGluY3JlYXNlZCByb290IHRvIHNob290IHJhdGlvIGFuZCBpbXByb3ZlZCBudXRyaWVudCB1cHRha2UgYW5kIHJlY292ZXJ5OyBwaHlzaW9sb2dpY2FsIHRyYWl0cywgZS5nLiwgc3RvbWF0YWwgY2xvc3VyZSwgZGVjcmVhc2VkIGxlYWYgcmVzcGlyYXRpb24sIGFuZCBtYWludGFpbmVkIHBob3Rvc3ludGhlc2lzOyBhbmQgYmlvY2hlbWljYWwgdHJhaXRzLCBlLmcuLCBpbmNyZWFzZWQgc2lnbmFsaW5nIG1vbGVjdWxlcyBhbmQgZW5oYW5jZWQgYW50aW94aWRhdGlvbiBlbnp5bWF0aWMgYWN0aXZpdHkuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjEwMDcvczEzNTkzLTAxNS0wMzM4LTYiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vbGluay5zcHJpbmdlci5jb20vY29udGVudC9wZGYvMTAuMTAwNy9zMTM1OTMtMDE1LTAzMzgtNi5wZGYiLCJpc3N1ZWREYXRlIjoiMjAxNS0xMi0xOCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkFncm9ub215IGZvciBTdXN0YWluYWJsZSBEZXZlbG9wbWVudCIsImlzc3VlIjoxLCJ2b2x1bWUiOjM2LCJpbXBhY3RGYWN0b3IiOiI5Ljc2In0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlNwcmluZ2VyIFNjaWVuY2UrQnVzaW5lc3MgTWVkaWEifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjE0OCwiY2l0ZWRCeUNvdW50Ijo1NTN9fSwicXVvdGUiOiJBZGRpdGlvbmFsbHksIHRoZSByZWFsbG9jYXRpb24gb2Ygd2F0ZXIgcmVzb3VyY2VzIGFuZCBtb2RpZmljYXRpb24gb2YgaXJyaWdhdGlvbiBzeXN0ZW1zIG1heSBoYXZlIGEgcm9sZSB0byBwbGF5IGZvciB0aGUgaW1wcm92ZW1lbnQgb2Ygd2F0ZXIgdXNlIGluIGFncmljdWx0dXJlIChIb21heW91bmZhciBldCBhbC4gMjAxNCkuIDcuMiBDaGFsbGVuZ2VzIFRoZSBjZW50cmFsIG5vdmVsdHkgb2YgUkRJIGlzIHRoYXQgaXQgYWxsb3dzIHBsYW50cyB0byBncm93IHVuZGVyIG1pbGQgd2F0ZXIgc3RyZXNzIHRvIGluZHVjZSByb290LXNvdXJjZWQgY2hlbWljYWwgc2lnLW5hbHMgZnJvbSBkcnlpbmcgcm9vdHMgdG8gcmVhY2ggdGhlIHNob290IHdoZXJlIHBoeXNpb2xvZ2ljYWwgYW5kIGJpb2NoZW1pY2FsIHByb2Nlc3NlcyBjYW4gYmUgcmVndWxhdGVkLiBIb3dldmVyLCB0aGUgcGh5c2lvbG9naWNhbCBhbmQgYmlvY2hlbWljYWwgcmVzcG9uc2VzIGFyZSBkaWZmaWN1bHQgdG8gcXVhbi10aWZ5LCBnaXZlbiB0aGF0IHBhcnRpYWwgcm9vdC16b25lIGlycmlnYXRpb24gaXMgdXN1YWxseSBhcHBsaWVkIHRvIHBsYW50cyBpbiBhY2NvcmRhbmNlIHdpdGggYm90aCB0ZW1wb3JhbCBhbmQgc3BhdGlhbCBkcm91Z2h0cy4gSW4gcHJhY3RpY2UsIGFkb3B0aW9uIG9mIHdhdGVyLXNhdmluZyB0ZWNobmlxdWUgcmVxdWlyZXMgc2NpLWVudGlmaWMgdW5kZXJzdGFuZGluZyBvZiB0aGUgbWVjaGFuaXNtcyBpbnZvbHZlZCBpbiB0aGUgcGh5c2lvbG9naWNhbCBhbmQgYmlvY2hlbWljYWwgcHJvY2Vzc2VzLiBBbHNvLCB0aGUgb3V0Y29tZSBvZiB0aGUgd2F0ZXIgc3RyZXNzLWluZHVjZWQgcmVzcG9uc2VzIGlzIGluZmx1ZW5jZWQgYnkgbWFueSBmYWN0b3JzLCBzdWNoIGFzIHRoZSBzdGFnZXMgb2YgY3JvcCBncm93dGggd2hlbiB0aGUgZGVmaWNpdCBpcyBhcHBsaWVkLCB0aGUgaW50ZW5zaXR5IGFuZCBzZXZlcml0eSBvZiBkZWZpY2l0IGltcG9zZWQsIHRoZSBtZXRob2Qgd2l0aCB3aGljaCBkZWZpY2l0IGlycmlnYXRpb24gaXMgYXBwbGllZCwgY3JvcCBzcGVjaWVzIGFuZCBjdWx0aXZhcnMsIGFuZCB0aGVpciByZXNwZWN0aXZlIG5vbi1jcml0aWNhbCBncm93dGggc3RhZ2VzIHdoZW4gdGhlIHN0YWdlLWJhc2VkIGRlZmljaXQgaXJyaWdhdGlvbiBtYXkgYmUgYXBwbGllZC4gVGhlc2UgZmFjdG9ycyBuZWVkIHRvIGJlIGludmVzdGlnYXRlZCBhdCBhIHN5c3RlbSBsZXZlbCB1bmRlciBzcGVjaWZpYyBncm93aW5nIGNvbmRpdGlvbnMuIFJESSBhbmQsIHBhcnRpY3VsYXJseSwgYWx0ZXJuYXRlIHBhcnRpYWwgcm9vdC16b25lIGlycmlnYXRpb24gYXJlIHN0aWxsIHJlbGF0aXZlbHkgbmV3IHRvIG1hbmFnZSBmYXJtIG1hbmFnZXJzLiBUaGlzIHRlY2gtbmlxdWUgaXMgbm90IGVhc3kgdG8gaW1wbGVtZW50IGluIGNyb3Agc3BlY2llcyB3aXRoIGRlbnNlIHBsYW50IHBvcHVsYXRpb24gc3VjaCBhcyBjZXJlYWwgYW5kIG9pbHNlZWQgY3JvcHMuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjY1LjU0MTQ4MTAxODA2NjQsImxlZnQiOjUxLjQzMTg0NjYxODY1MjM0NCwid2lkdGgiOjM5Ljk5NDI4MTc2ODc5ODgzLCJoZWlnaHQiOjEuMzY0MjYyNzAwMDgwODcxNn19LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjo2Ny4xMTg2NDQ3MTQzNTU0NywibGVmdCI6NTEuNDMxODQ2NjE4NjUyMzQ0LCJ3aWR0aCI6MzkuOTk1OTQxMTYyMTA5Mzc1LCJoZWlnaHQiOjEuMzY0MjYyNzAwMDgwODcxNn19LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjo2OC42ODgyNDAwNTEyNjk1MywibGVmdCI6NTEuNDMxODQ2NjE4NjUyMzQ0LCJ3aWR0aCI6MzkuOTkyNTgwNDEzODE4MzYsImhlaWdodCI6MS4zNjQyNjI3MDAwODA4NzE2fX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjcwLjI2NTM5NjExODE2NDA2LCJsZWZ0Ijo1MS40MzE4NDY2MTg2NTIzNDQsIndpZHRoIjoxNy44OTA4OTIwMjg4MDg1OTQsImhlaWdodCI6MS4zNjQyNjI3MDAwODA4NzE2fX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjczLjQwNzEwNDQ5MjE4NzUsImxlZnQiOjUxLjQzMTg0NjYxODY1MjM0NCwid2lkdGgiOjEwLjEwMzU5MTkxODk0NTMxMiwiaGVpZ2h0IjoxLjM3Njg1NzYzODM1OTA2OTh9fSx7InBhZ2VOdW1iZXIiOjEzLCJiYm94Ijp7InRvcCI6NzYuNTY2NDU5NjU1NzYxNzIsImxlZnQiOjUxLjQzMTg0NjYxODY1MjM0NCwid2lkdGgiOjQwLjAwNzY3MTM1NjIwMTE3LCJoZWlnaHQiOjEzLjk1ODgyMDM0MzAxNzU3OH19LHsicGFnZU51bWJlciI6MTQsImJib3giOnsidG9wIjo3LjI4NDg1ODcwMzYxMzI4MSwibGVmdCI6OC41NzE0MTg3NjIyMDcwMzEsIndpZHRoIjo0MC4wMDc2NDA4Mzg2MjMwNSwiaGVpZ2h0IjoxLjM2NDI2MzE3NjkxODAyOTh9fSx7InBhZ2VOdW1iZXIiOjE0LCJiYm94Ijp7InRvcCI6OC44NTQ0NTMwODY4NTMwMjcsImxlZnQiOjguNTcxNDE4NzYyMjA3MDMxLCJ3aWR0aCI6NDAuMDE1OTk1MDI1NjM0NzY2LCJoZWlnaHQiOjEuMzY0MjYzNjUzNzU1MTg4fX0seyJwYWdlTnVtYmVyIjoxNCwiYmJveCI6eyJ0b3AiOjEwLjQzMTYxMTA2MTA5NjE5MSwibGVmdCI6OC41NzE0MTg3NjIyMDcwMzEsIndpZHRoIjo0MC4wMDc2NDA4Mzg2MjMwNSwiaGVpZ2h0IjoxLjM2NDI2MzY1Mzc1NTE4OH19LHsicGFnZU51bWJlciI6MTQsImJib3giOnsidG9wIjoxMi4wMDg3NjE0MDU5NDQ4MjQsImxlZnQiOjguNTcxNDE4NzYyMjA3MDMxLCJ3aWR0aCI6NDAuMDA0MzAyOTc4NTE1NjI1LCJoZWlnaHQiOjEuMzY0MjYzNjUzNzU1MTg4fX0seyJwYWdlTnVtYmVyIjoxNCwiYmJveCI6eyJ0b3AiOjEzLjU3ODM1NTc4OTE4NDU3LCJsZWZ0Ijo4LjU3MTQxODc2MjIwNzAzMSwid2lkdGgiOjM5Ljk5NzU5Njc0MDcyMjY1NiwiaGVpZ2h0IjoxLjM2NDI2MzY1Mzc1NTE4OH19LHsicGFnZU51bWJlciI6MTQsImJib3giOnsidG9wIjoxNS4xNTU1MTM3NjM0Mjc3MzQsImxlZnQiOjguNTcxNDE4NzYyMjA3MDMxLCJ3aWR0aCI6NDAuMDEwOTkzOTU3NTE5NTMsImhlaWdodCI6MS4zNjQyNjM2NTM3NTUxODh9fSx7InBhZ2VOdW1iZXIiOjE0LCJiYm94Ijp7InRvcCI6MTYuNzMyNjY0MTA4Mjc2MzY3LCJsZWZ0Ijo4LjU3MTQxODc2MjIwNzAzMSwid2lkdGgiOjQwLjAxMDk5MDE0MjgyMjI2NiwiaGVpZ2h0IjoxLjM2NDI2MjcwMDA4MDg3MTZ9fSx7InBhZ2VOdW1iZXIiOjE0LCJiYm94Ijp7InRvcCI6MTguMzAyMjU5NDQ1MTkwNDMsImxlZnQiOjguNTcxNDE4NzYyMjA3MDMxLCJ3aWR0aCI6NDAuMDA3NjM3MDIzOTI1NzgsImhlaWdodCI6MS4zNjQyNjI3MDAwODA4NzE2fX0seyJwYWdlTnVtYmVyIjoxNCwiYmJveCI6eyJ0b3AiOjE5Ljg3OTQxNzQxOTQzMzU5NCwibGVmdCI6OC41NzE0MTg3NjIyMDcwMzEsIndpZHRoIjoxMy4wODkzMDQ5MjQwMTEyMywiaGVpZ2h0IjoxLjM2NDI2MjcwMDA4MDg3MTZ9fSx7InBhZ2VOdW1iZXIiOjE0LCJiYm94Ijp7InRvcCI6MjEuNDU2NTY3NzY0MjgyMjI3LCJsZWZ0IjoxMC40NzU5ODc0MzQzODcyMDcsIndpZHRoIjozOC4xMTE0MjczMDcxMjg5MDYsImhlaWdodCI6MS4zNjQyNjI3MDAwODA4NzE2fX0seyJwYWdlTnVtYmVyIjoxNCwiYmJveCI6eyJ0b3AiOjIzLjAyNjE2MzEwMTE5NjI5LCJsZWZ0Ijo4LjU3MTQxODc2MjIwNzAzMSwid2lkdGgiOjM5Ljk5MjU4ODA0MzIxMjg5LCJoZWlnaHQiOjEuMzY0MjYyNzAwMDgwODcxNn19LHsicGFnZU51bWJlciI6MTQsImJib3giOnsidG9wIjoyNC42MDMzMjEwNzU0Mzk0NTMsImxlZnQiOjguNTcxNDE4NzYyMjA3MDMxLCJ3aWR0aCI6NDAuMDAyNjEzMDY3NjI2OTUsImhlaWdodCI6MS4zNjQyNjI3MDAwODA4NzE2fX0seyJwYWdlTnVtYmVyIjoxNCwiYmJveCI6eyJ0b3AiOjI2LjE4MDQ2OTUxMjkzOTQ1MywibGVmdCI6OC41NzE0MTg3NjIyMDcwMzEsIndpZHRoIjozOS45OTU5MjIwODg2MjMwNSwiaGVpZ2h0IjoxLjM2NDI2MjcwMDA4MDg3MTZ9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIxNCJ9fV0%3D. Holistic approaches combining crop selection, timing, and mulching maximize transpiration efficiency, supporting sustainable intensification and reducing deforestation . Collaborative frameworks are vital to balance agricultural needs with environmental protection (Mansoor et al., 2025)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjoxLCJ3b3JrIjp7InR5cGUiOjMwLCJzdWJ0eXBlIjp7fSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2hlaWtoIiwiZmFtaWx5IjoiTWFuc29vciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTaGFoemFkIiwiZmFtaWx5IjoiSXFiYWwifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2ltb25hIE1hcmlhbmEiLCJmYW1pbHkiOiJQb3Blc2N1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNvbmcgTGltIiwiZmFtaWx5IjoiS2ltIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNoZWlraCIsImZhbWlseSI6Ik1hbnNvb3IifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSmVvbmdobyIsImZhbWlseSI6IkJhZWsifX1dLCJpZHMiOnsiaXNzbiI6WyIxNjY0LTQ2MlgiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzODkvZnBscy4yMDI1LjE1ODc4NjkiLCJwbWlkIjoiNDA0Mzg3MzciLCJvcGVuYWxleCI6Ilc0NDEwMzc4Mjg5IiwidXBsb2FkSWQiOiJkRE9WVGVxS2h1ajV5aDFWTVhEUSJ9LCJ0aXRsZSI6IkludGVncmF0aW9uIG9mIHNtYXJ0IHNlbnNvcnMgYW5kIElPVCBpbiBwcmVjaXNpb24gYWdyaWN1bHR1cmU6IHRyZW5kcywgY2hhbGxlbmdlcyBhbmQgZnV0dXJlIHByb3NwZWN0aXZlcyIsImFic3RyYWN0IjoiVHJhZGl0aW9uYWwgZmFybWluZyBtZXRob2RzLCBlZmZlY3RpdmUgZm9yIGdlbmVyYXRpb25zLCBzdHJ1Z2dsZSB0byBtZWV0IHJpc2luZyBnbG9iYWwgZm9vZCBkZW1hbmRzIGR1ZSB0byBsaW1pdGF0aW9ucyBpbiBwcm9kdWN0aXZpdHksIGVmZmljaWVuY3ksIGFuZCBzdXN0YWluYWJpbGl0eSBhbWlkIGNsaW1hdGUgY2hhbmdlIGFuZCByZXNvdXJjZSBzY2FyY2l0eS4gUHJlY2lzaW9uIGFncmljdWx0dXJlIHByZXNlbnRzIGEgdmlhYmxlIHNvbHV0aW9uIGJ5IG9wdGltaXppbmcgcmVzb3VyY2UgdXNlLCBlbmhhbmNpbmcgZWZmaWNpZW5jeSwgYW5kIGZvc3RlcmluZyBzdXN0YWluYWJsZSBwcmFjdGljZXMgdGhyb3VnaCBkYXRhLWRyaXZlbiBkZWNpc2lvbi1tYWtpbmcgc3VwcG9ydGVkIGJ5IGFkdmFuY2VkIHNlbnNvcnMgYW5kIEludGVybmV0IG9mIFRoaW5ncyAoSW9UKSB0ZWNobm9sb2dpZXMuIFRoaXMgcmV2aWV3IGV4YW1pbmVzIHZhcmlvdXMgc21hcnQgc2Vuc29ycyB1c2VkIGluIHByZWNpc2lvbiBhZ3JpY3VsdHVyZSwgaW5jbHVkaW5nIHNvaWwgc2Vuc29ycyBmb3IgbW9pc3R1cmUsIHBILCBhbmQgcGxhbnQgc3RyZXNzIHNlbnNvcnMgZXRjLiBUaGVzZSBzZW5zb3JzIGRlbGl2ZXIgcmVhbC10aW1lIGRhdGEgdGhhdCBlbmFibGVzIGluZm9ybWVkIGRlY2lzaW9uLW1ha2luZywgZmFjaWxpdGF0aW5nIHRhcmdldGVkIGludGVydmVudGlvbnMgbGlrZSBvcHRpbWl6ZWQgaXJyaWdhdGlvbiwgZmVydGlsaXphdGlvbiwgYW5kIHBlc3QgbWFuYWdlbWVudC4gQWRkaXRpb25hbGx5LCB0aGUgcmV2aWV3IGhpZ2hsaWdodHMgdGhlIHRyYW5zZm9ybWF0aXZlIHJvbGUgb2YgSW9UIGluIHByZWNpc2lvbiBhZ3JpY3VsdHVyZS4gVGhlIGludGVncmF0aW9uIG9mIHNlbnNvciBuZXR3b3JrcyB3aXRoIElvVCBwbGF0Zm9ybXMgYWxsb3dzIGZvciByZW1vdGUgbW9uaXRvcmluZywgZGF0YSBhbmFseXNpcyB2aWEgYXJ0aWZpY2lhbCBpbnRlbGxpZ2VuY2UgKEFJKSBhbmQgbWFjaGluZSBsZWFybmluZyAoTUwpLCBhbmQgYXV0b21hdGVkIGNvbnRyb2wgc3lzdGVtcywgZW5hYmxpbmcgcHJlZGljdGl2ZSBhbmFseXRpY3MgdG8gYWRkcmVzcyBjaGFsbGVuZ2VzIHN1Y2ggYXMgZGlzZWFzZSBvdXRicmVha3MgYW5kIHlpZWxkIGZvcmVjYXN0aW5nLiBIb3dldmVyLCB3aGlsZSBwcmVjaXNpb24gYWdyaWN1bHR1cmUgb2ZmZXJzIHNpZ25pZmljYW50IGJlbmVmaXRzLCBpdCBmYWNlcyBjaGFsbGVuZ2VzIGluY2x1ZGluZyBoaWdoIGluaXRpYWwgaW52ZXN0bWVudCBjb3N0cywgY29tcGxleGl0aWVzIGluIGRhdGEgbWFuYWdlbWVudCwgbmVlZHMgZm9yIHRlY2huaWNhbCBleHBlcnRpc2UsIGRhdGEgc2VjdXJpdHkgYW5kIHByaXZhY3kgY29uY2VybnMsIGFuZCBpc3N1ZXMgd2l0aCBjb25uZWN0aXZpdHkgaW4gcmVtb3RlIGFncmljdWx0dXJhbCBhcmVhcy4gQWRkcmVzc2luZyB0aGVzZSB0ZWNobm9sb2dpY2FsIGFuZCBlY29ub21pYyBjaGFsbGVuZ2VzIGlzIGVzc2VudGlhbCBmb3IgbWF4aW1pemluZyB0aGUgcG90ZW50aWFsIG9mIHByZWNpc2lvbiBhZ3JpY3VsdHVyZSBpbiBlbmhhbmNpbmcgZ2xvYmFsIGZvb2Qgc2VjdXJpdHkgYW5kIHN1c3RhaW5hYmlsaXR5LiBUaGVyZWZvcmUsIGluIHRoaXMgcmV2aWV3IHdlIGV4cGxvcmUgdGhlIGxhdGVzdCB0cmVuZHMsIGNoYWxsZW5nZXMsIGFuZCBvcHBvcnR1bml0aWVzIGFzc29jaWF0ZWQgd2l0aCBJb1QgZW5hYmxlZCBzbWFydCBzZW5zb3JzIGluIHByZWNpc2lvbiBhZ3JpY3VsdHVyZS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMzM4OS9mcGxzLjIwMjUuMTU4Nzg2OSIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cuZnJvbnRpZXJzaW4ub3JnL2pvdXJuYWxzL3BsYW50LXNjaWVuY2UvYXJ0aWNsZXMvMTAuMzM4OS9mcGxzLjIwMjUuMTU4Nzg2OS9wZGYiLCJpc3N1ZWREYXRlIjoiMjAyNS0wNS0xNCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTEiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZyb250aWVycyBpbiBQbGFudCBTY2llbmNlIiwidm9sdW1lIjoxNiwiaW1wYWN0RmFjdG9yIjoiNy4wMCJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjE1ODc4NjksImxhc3RQYWdlIjoxNTg3ODY5fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkZyb250aWVycyBNZWRpYSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MTk2LCJjaXRlZEJ5Q291bnQiOjI1fX0sInF1b3RlIjoiUHJlY2lzaW9uIGFncmljdWx0dXJlIGlzIHRyYW5zZm9ybWluZyBhZ3JpY3VsdHVyZSBieSBpbnRlZ3JhdGluZyBhZHZhbmNlZCB0ZWNobm9sb2dpZXMgdG8gbWVldCBmb29kIHByb2R1Y3Rpb24gZGVtYW5kcy4gVGhpcyBzaGlmdCBmcm9tIGEgZGF0YS1kcml2ZW4gbW9kZWwgdG8gYSBjb2xsYWJvcmF0aXZlIGFwcHJvYWNoLCBpbmNvcnBvcmF0aW5nIG11bHRpLW9taWNzIGRhdGEgYW5hbHlzaXMsIGVtcGhhc2l6ZXMgdGhlIHN5bmVyZ3kgYmV0d2VlbiB0ZWNobm9sb2d5IGFuZCBodW1hbiBleHBlcnRpc2UsIGZvc3RlcmluZyByZXNpbGllbnQgYWdyaWN1bHR1cmFsIHN5c3RlbXMgKEFiaW95ZSBldCBhbC4sIDIwMjMpLiBBcyBzdGFrZWhvbGRlcnMgYWRvcHQgdGhlc2UgaW5ub3ZhdGlvbnMsIGluY2x1ZGluZyB0aGUgaW50ZWdyYXRpb24gb2YgbXVsdGktb21pY3MgaW5zaWdodHMsIHRoZSBhZ3JpY3VsdHVyYWwgc2VjdG9yIGlzIGluY3JlYXNpbmdseSBwb2lzZWQgdG8gZW5zdXJlIGZvb2Qgc2VjdXJpdHkgYW5kIHN1c3RhaW5hYmlsaXR5IGZvciB0aGUgZ3Jvd2luZyBnbG9iYWwgcG9wdWxhdGlvbi4gQXMgZW52aXJvbm1lbnRhbCBjb25jZXJucyBncm93LCB0aGVyZSBpcyBhbiBpbmNyZWFzaW5nIGVtcGhhc2lzIG9uIGFncmljdWx0dXJhbCBwcmFjdGljZXMgdGhhdCBlbmhhbmNlIHlpZWxkIHdoaWxlIG1haW50YWluaW5nIGVjb2xvZ2ljYWwgYmFsYW5jZS4gQnkgaW50ZWdyYXRpbmcgc3VzdGFpbmFibGUgdGVjaG5pcXVlcyB3aXRoaW4gdGhlIHByZWNpc2lvbiBhZ3JpY3VsdHVyZSBmcmFtZXdvcmssIGZhcm1lcnMgY2FuIGVmZmVjdGl2ZWx5IHJlZHVjZSB0aGVpciBjYXJib24gZm9vdHByaW50LCBib2xzdGVyIGJpb2RpdmVyc2l0eSwgYW5kIHByb21vdGUgaGVhbHRoaWVyIHNvaWxzIChLb25mbyBldCBhbC4sIDIwMjQpLiBQcmVjaXNpb24gYWdyaWN1bHR1cmUgb3B0aW1pemVzIHJlc291cmNlIHVzZSBhbmQgbWluaW1pemVzIHdhc3RlLCBsZWFkaW5nIHRvIHNpZ25pZmljYW50IGVudmlyb25tZW50YWwgYmVuZWZpdHMuIEZvciBpbnN0YW5jZSwgcmVkdWN0aW9ucyBpbiBjYXJib24gZW1pc3Npb25zIHJlc3VsdCBmcm9tIGRlY3JlYXNlZCBmZXJ0aWxpemVyIGFuZCBwZXN0aWNpZGUgYXBwbGljYXRpb24sIHdoaWNoIG5vdCBvbmx5IGxvd2VycyBncmVlbmhvdXNlIGdhcyBlbWlzc2lvbnMgYnV0IGFsc28gZW5oYW5jZXMgc29pbCBoZWFsdGgsIG1lYXN1cmFibGUgdGhyb3VnaCBhIHNvaWwgaGVhbHRoIGluZGV4IHRoYXQgcmVmbGVjdHMgaW1wcm92ZWQgb3JnYW5pYyBtYXR0ZXIgYW5kIG51dHJpZW50IGF2YWlsYWJpbGl0eSAoQWhtYWQgYW5kIERhciwgMjAyMDsgRmFyb29xdWkgZXQgYWwuLCAyMDI0KS4gTW9yZW92ZXIsIHByZWNpc2lvbiBpcnJpZ2F0aW9uIGVuaGFuY2VzIHdhdGVyIHVzZSBlZmZpY2llbmN5LCBjb25zZXJ2aW5nIHZpdGFsIHJlc291cmNlcyBhbmQgcHJvdGVjdGluZyBsb2NhbCBlY29zeXN0ZW1zLiBDb2xsZWN0aXZlbHksIHRoZXNlIG1ldHJpY3MgaWxsdXN0cmF0ZSBob3cgcHJlY2lzaW9uIGFncmljdWx0dXJlIGZvc3RlcnMgc3VzdGFpbmFibGUgZmFybWluZyBtZXRob2RzIGFuZCBlbmhhbmNlcyBvdmVyYWxsIGVudmlyb25tZW50YWwgc3Rld2FyZHNoaXAuICIsInNjb3JlIjowLjE4MjMwMDAwMTM4MjgyNzc2LCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjQ2Ljc0ODgwMjE4NTA1ODU5NCwibGVmdCI6OC4zMzE1OTkyMzU1MzQ2NjgsIndpZHRoIjo0MC40ODIxNDcyMTY3OTY4NzUsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6NDguMjMwNjU1NjcwMTY2MDE2LCJsZWZ0Ijo4LjMzMTU5OTIzNTUzNDY2OCwid2lkdGgiOjQwLjQ5MDE2OTUyNTE0NjQ4NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo0OS43MTkyNDIwOTU5NDcyNjYsImxlZnQiOjguMzMxNTk5MjM1NTM0NjY4LCJ3aWR0aCI6NDAuNDg2NTQ1NTYyNzQ0MTQsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6NTEuMjAxMDkxNzY2MzU3NDIsImxlZnQiOjguMzMxNTk5MjM1NTM0NjY4LCJ3aWR0aCI6NDAuNDg4NjY2NTM0NDIzODMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6NTIuNjg5NjgyMDA2ODM1OTQsImxlZnQiOjguMzMxNTk5MjM1NTM0NjY4LCJ3aWR0aCI6NDAuNDkzMDQ1ODA2ODg0NzY2LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjU0LjE3MTUzMTY3NzI0NjA5NCwibGVmdCI6OC4zMzE1OTkyMzU1MzQ2NjgsIndpZHRoIjo0MC40ODU5MzkwMjU4Nzg5MDYsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6NTUuNjYwMTIxOTE3NzI0NjEsImxlZnQiOjguMzMxNTk5MjM1NTM0NjY4LCJ3aWR0aCI6NDAuNDkwMDEzMTIyNTU4NTk0LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjU3LjE0MTk3MTU4ODEzNDc2NiwibGVmdCI6OC4zMzE2MDAxODkyMDg5ODQsIndpZHRoIjo0MC40ODQ1NzMzNjQyNTc4MSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo1OC42MzA1NjE4Mjg2MTMyOCwibGVmdCI6OC4zMzE2MDAxODkyMDg5ODQsIndpZHRoIjo0MC40OTA0NzA4ODYyMzA0NywiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo2MC4xMTI0MTUzMTM3MjA3LCJsZWZ0Ijo4LjMzMTYwMDE4OTIwODk4NCwid2lkdGgiOjE4LjcwMjYyNzE4MjAwNjgzNiwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo2MS41OTQyNzI2MTM1MjUzOSwibGVmdCI6MTAuNjgzNTI1MDg1NDQ5MjE5LCJ3aWR0aCI6MzguMTQwNTE4MTg4NDc2NTYsImhlaWdodCI6MS4xODA1MTY5NTgyMzY2OTQzfX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6NjMuMDgyODYyODU0MDAzOTA2LCJsZWZ0Ijo4LjMzMTYwMDE4OTIwODk4NCwid2lkdGgiOjQwLjQ4MjE1MTAzMTQ5NDE0LCJoZWlnaHQiOjEuMTgwNTE2OTU4MjM2Njk0M319LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjY0LjU2NDcxMjUyNDQxNDA2LCJsZWZ0Ijo4LjMzMTYwMDE4OTIwODk4NCwid2lkdGgiOjQwLjQ5MjI3NTIzODAzNzExLCJoZWlnaHQiOjEuMTgwNTE2OTU4MjM2Njk0M319LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjY2LjA1MzI5ODk1MDE5NTMxLCJsZWZ0Ijo4LjMzMTYwMDE4OTIwODk4NCwid2lkdGgiOjQwLjQ5NDg2NTQxNzQ4MDQ3LCJoZWlnaHQiOjEuMTgwNTE2OTU4MjM2Njk0M319LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjY3LjUzNTE0ODYyMDYwNTQ3LCJsZWZ0Ijo4LjMzMTYwMDE4OTIwODk4NCwid2lkdGgiOjQwLjQ4NDQzNjAzNTE1NjI1LCJoZWlnaHQiOjEuMTgwNTE2OTU4MjM2Njk0M319LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjY5LjAyMzc0MjY3NTc4MTI1LCJsZWZ0Ijo4LjMzMTYwMDE4OTIwODk4NCwid2lkdGgiOjQwLjUwMzYzMTU5MTc5Njg3NSwiaGVpZ2h0IjoxLjE4MDUxNjk1ODIzNjY5NDN9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo3MC41MDU1OTIzNDYxOTE0LCJsZWZ0Ijo4LjMzMTU5OTIzNTUzNDY2OCwid2lkdGgiOjQwLjQ4Mjc2NTE5Nzc1MzkwNiwiaGVpZ2h0IjoxLjE4MDUxNjk1ODIzNjY5NDN9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo3MS45OTQxODY0MDEzNjcxOSwibGVmdCI6OC4zMzE1OTkyMzU1MzQ2NjgsIndpZHRoIjo0MC40ODg4MDM4NjM1MjUzOSwiaGVpZ2h0IjoxLjE4MDUxNjk1ODIzNjY5NDN9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo3My40NzYwMzYwNzE3NzczNCwibGVmdCI6OC4zMzE0NDc2MDEzMTgzNiwid2lkdGgiOjQwLjQ3Njg2Mzg2MTA4Mzk4NCwiaGVpZ2h0IjoxLjE4MDUxNjk1ODIzNjY5NDN9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo3NC45NjQ2MjI0OTc1NTg2LCJsZWZ0Ijo4LjMzMTQ0NzYwMTMxODM2LCJ3aWR0aCI6NDAuNDg1Nzg2NDM3OTg4MjgsImhlaWdodCI6MS4xODA1MTY5NTgyMzY2OTQzfX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6NzYuNDQ2NDcyMTY3OTY4NzUsImxlZnQiOjguMzMxNDQ3NjAxMzE4MzYsIndpZHRoIjo0MC40ODc2MDIyMzM4ODY3MiwiaGVpZ2h0IjoxLjE4MDUxNjk1ODIzNjY5NDN9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo3Ny45MzUwNjYyMjMxNDQ1MywibGVmdCI6OC4zMzE0NDc2MDEzMTgzNiwid2lkdGgiOjQwLjQ4Mzk4MjA4NjE4MTY0LCJoZWlnaHQiOjEuMTgwNTE2OTU4MjM2Njk0M319LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjc5LjQxNjkxNTg5MzU1NDY5LCJsZWZ0Ijo4LjMzMTI5NjkyMDc3NjM2Nywid2lkdGgiOjQwLjQ4OTg3MTk3ODc1OTc2NiwiaGVpZ2h0IjoxLjE4MDUxNjk1ODIzNjY5NDN9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo4MC45MDU1MDIzMTkzMzU5NCwibGVmdCI6OC4zMzEyOTk3ODE3OTkzMTYsIndpZHRoIjo0MC40ODczMDQ2ODc1LCJoZWlnaHQiOjEuMTgwNTE2OTU4MjM2Njk0M319LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjgyLjM4NzM1MTk4OTc0NjEsImxlZnQiOjguMzMxMzAxNjg5MTQ3OTUsIndpZHRoIjo0MC40OTEyMjYxOTYyODkwNiwiaGVpZ2h0IjoxLjE4MDUxNjk1ODIzNjY5NDN9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo4My44NzU5NDYwNDQ5MjE4OCwibGVmdCI6OC4zMzEzMDE2ODkxNDc5NSwid2lkdGgiOjQwLjQ4NjgzMTY2NTAzOTA2LCJoZWlnaHQiOjEuMTgwNTE2OTU4MjM2Njk0M319LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjg1LjM1Nzc4ODA4NTkzNzUsImxlZnQiOjguMzMxMzAxNjg5MTQ3OTUsIndpZHRoIjo0MC40ODIxNTg2NjA4ODg2NywiaGVpZ2h0IjoxLjE4MDUxNjk1ODIzNjY5NDN9fV19LCJjaHVua0lkIjoiY2I2YzUxNzgtNjJmMy00OTg4LWJmYWUtYTNjNTU3Mzg0NDZkIiwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiMTU4Nzg3MSJ9fV0%3D.

### Table 12: Economic Factors for Modernization

| Factor | Barrier/Opportunity | Mitigation Strategy |
| --- | --- | --- |
| Initial Capital | High installation cost | Subsidies and incentives |
| Operational ROI | Long-term savings | Energy and water reductions |
| Maintenance | Technical skill required | Training programs for farmers |
| System Longevity | 10–20 years (typical) | High-quality component use |
| Labor Costs | Automated reduction | IoT integration |
| Resource Fees | Rising water prices | High-efficiency adoption |
| Market Value | Better crop quality | Access to premium markets |
| Scalability | Small vs Large fields | Modular system design |

### Research Gaps and Emerging Technologies

Key gaps include system-level studies on RDI under varying deficits, crop stages, and dense plantings, alongside limited quantification of biotech water savings (Chai et al., 2015; Chami & Moujabber, 2016)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkRhbmllbCBFbCIsImZhbWlseSI6IkNoYW1pIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1hcm91biBFbCIsImZhbWlseSI6Ik1vdWphYmJlciJ9fV0sImlkcyI6eyJpc3NuIjpbIjAwMzgtMjM1MyIsIjE5OTYtNzQ4OSJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMTcxNTkvc2Fqcy4yMDE2LzIwMTUwNDU3IiwibWFnIjoiMjUyMzY3OTY5OCIsIm9wZW5hbGV4IjoiVzI1MjM2Nzk2OTgifSwidGl0bGUiOiJEcm91Z2h0LCBjbGltYXRlIGNoYW5nZSBhbmQgc3VzdGFpbmFiaWxpdHkgb2Ygd2F0ZXIgaW4gYWdyaWN1bHR1cmU6IEEgcm9hZG1hcCB0b3dhcmRzIHRoZSBOV1JTMiIsImFic3RyYWN0IjoiVGhlIGZyZXF1ZW5jeSBhbmQgaW50ZW5zaXR5IG9mIGRyb3VnaHQsIGV4dHJlbWUgZXZlbnRzIGFuZCBoaWdoIHdpbmQgdmVsb2NpdGllcyBpbiBTb3V0aCBBZnJpY2EgYXJlIGV4cGVjdGVkIHRvIGluY3JlYXNlIGluIHRoZSBuZXh0IGNlbnR1cnkgYXMgYSByZXN1bHQgb2YgdGhlIGNoYW5naW5nIGNsaW1hdGUuIFRoZSBOYXRpb25hbCBXYXRlciBSZXNvdXJjZSBTdHJhdGVneSAyIChOV1JTMikgaGFzIHNldCBvdXQgdGhlIGdlbmVyYWwgYW5kIHN0cmF0ZWdpYyBkaXJlY3Rpb25zIGZvciB3YXRlciByZXNvdXJjZXMgbWFuYWdlbWVudCBpbiB0aGUgY291bnRyeSBmb3IgdGhlIG5leHQgMjAgeWVhcnMuIEhvd2V2ZXIsIHRoZSBzdHJhdGVneSBkb2VzIG5vdCBkcmF3IGEgZnJhbWV3b3JrIHRhaWxvcmVkIHNwZWNpZmljYWxseSBmb3IgYWdyaWN1bHR1cmFsIHVzZSwgd2l0aCBzcGVjaWZpYyBtZWFzdXJlcyBhbmQgZ29hbHMuIFRoZXJlZm9yZSwgdG8gcmVhY2ggc3VzdGFpbmFiaWxpdHkgb2Ygd2F0ZXIgaW4gYWdyaWN1bHR1cmUsIGZvdXIgbWFqb3Igc3RyYXRlZ2ljIGdvYWxzIGFyZSBzdWdnZXN0ZWQsIG9uIHdoaWNoIHJlc2VhcmNoIGluc3RpdHV0aW9ucyBjYW4gZm9jdXMgYW5kIHByb21vdGUgdGhyb3VnaCBnb29kIGdvdmVybmFuY2UuIFRoZSBzdHJhdGVneSBlbXBoYXNpc2VzOiAoMSkgY3JvcCByZXNlYXJjaCB0byBmaW5kIG5ldyBkcm91Z2h0LWFuZCBoZWF0LSB0b2xlcmFudCBhbmQgcmVzaXN0YW50IGJyZWVkcyBhbmQgdmFyaWV0aWVzOyAoMikgaW50ZW5zaWZpZWQgcmVzZWFyY2ggaW4gYWdyaWN1bHR1cmFsIHByYWN0aWNlczsgKDMpIGluY3JlYXNpbmcgdGhlIGVmZmljaWVuY3kgb2Ygd2F0ZXIgdXNlIHdpdGhpbiBhZ3JpY3VsdHVyZTsgYW5kICg0KSBpbnRlZ3JhdGluZyBhbGwgdGhlc2Ugc3RyYXRlZ2ljIGdvYWxzIHdpdGhpbiBhIHN1c3RhaW5hYmxlIHJlc2VhcmNoIGZyYW1ld29yay4gRmluYWxseSwgdGhlIHJlc2VhcmNoIGNhbGxzIGZvciByYXBpZCBhY3Rpb24gYW5kIGltcGxlbWVudGF0aW9uLiBTaWduaWZpY2FuY2U6IFRoZSBmcmFtZXdvcmsgaXMgcHJvcG9zZWQgZm9yIHN0YWtlaG9sZGVycyBhbmQgcG9saWN5bWFrZXJzIGluIGhpZ2hlciBlZHVjYXRpb24sIGFncmljdWx0dXJlIGFuZCByZXNvdXJjZXMgbWFuYWdlbWVudCBpbiBTb3V0aCBBZnJpY2EgZm9yIG5ldyByZXNlYXJjaCBob3Jpem9ucyBhdCBuYXRpb25hbCBsZXZlbCB0byBpbXByb3ZlIG92ZXJhbGwgYWdyaWN1bHR1cmFsIHN1c3RhaW5hYmlsaXR5IGJ5IDIwMzAgYXMgc3RpcHVsYXRlZCBieSB0aGUgTWlsbGVubml1bSBEZXZlbG9wbWVudCBHb2Fscy4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTcxNTkvc2Fqcy4yMDE2LzIwMTUwNDU3IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5zYWpzLmNvLnphL2FydGljbGUvZG93bmxvYWQvMzY2My80NzY5IiwiaXNzdWVkRGF0ZSI6IjIwMTYtMDktMjMiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA5IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJTb3V0aCBBZnJpY2FuIEpvdXJuYWwgb2YgU2NpZW5jZSIsInZvbHVtZSI6MTEyLCJpbXBhY3RGYWN0b3IiOiIxLjQyIn0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6NCwibGFzdFBhZ2UiOjR9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiQWNhZGVteSBvZiBTY2llbmNlIG9mIFNvdXRoIEFmcmljYSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MzksImNpdGVkQnlDb3VudCI6MjV9fSwicXVvdGUiOiJJbiB0aGUgbWVhbnRpbWUsIGNyb3AgYnJlZWRpbmcgKGVzcGVjaWFsbHkgdGhlICBjb252ZW50aW9uYWwgYnJlZWRpbmcgdXNpbmcgbWFya2VyLWFzc2lzdGVkIHNlbGVjdGlvbikgaXMgYW4gZXh0cmVtZWx5ICB1c2VmdWwgdGVjaG5pcXVlIGZvciBicmVlZGluZyBjb21wbGV4IHRyYWl0cyAoc3VjaCBhcyBkcm91Z2h0IHRvbGVyYW5jZSkgIGludG8gaW1wcm92ZWQgdmFyaWV0aWVzLjIyIFRoZXJlIGlzIGEgZGVhcnRoIG9mIHN0dWRpZXMgcmVwb3J0ZWQgaW4gdGhlIGxpdGVyYXR1cmUgdG8gZW5hYmxlIHF1YW50aS1maWNhdGlvbiBvZiB3YXRlciBjb25zdW1wdGlvbiBhbmQgc2F2aW5ncyBhcyBhIHJlc3VsdCBvZiBzdWNoIGltcHJvdmVkIHZhcmlldGllcy4gSG93ZXZlciwgYSBzdHVkeSBoYXMgc2hvd24gdGhhdCBkcm91Z2h0LXRvbGVyYW50IGFuZCBkcm91Z2h0LXJlc2lzdGFudCB0cmFpdHMgY291bGQgaW1wcm92ZSB0aGUgeWllbGQgb2YgbWFpemUgaW4gQWZyaWNhIGJ5IHVwIHRvIDMwJSAgdW5kZXIgZHJvdWdodCBjb25kaXRpb25zIHdoZW4gY29tcGFyZWQgd2l0aCBjb21tZXJjaWFsIHNlZWRzLjIzICBGdXJ0aGVybW9yZSwgR3VyIGFuZCBaYW1pcjI0IHN1Y2Nlc3NmdWxseSBvYnRhaW5lZCA1MCUgaGlnaGVyIHlpZWxkcyBvZiAgdG9tYXRvLCBldmVuIHVuZGVyIGRyb3VnaHQgY29uZGl0aW9ucywgYnkgYnJlZWRpbmcgY3VsdGl2YXRlZCB0b21hdG9lcyAgd2l0aCB3aWxkIHJlbGF0aXZlcy4gRGVyeW5nIGV0IGFsLjI1IGZvdW5kIOKAkyBvbiBhIGdsb2JhbCBzY2FsZSDigJMgdGhhdCBieSB0aGUgIDIwODBzLCBleHRyZW1lIGhlYXQgc3RyZXNzIGF0IGFudGhlc2lzIGNvdWxkOiAoMSkgZG91YmxlIGxvc3NlcyBvZiBtYWl6ZSAgeWllbGQsICgyKSByZWR1Y2UgcHJvamVjdGVkIGdhaW5zIGluIHNwcmluZyB3aGVhdCB5aWVsZCBieSBoYWxmLCBhbmQgKDMpICByZWR1Y2UgcHJvamVjdGVkIGdhaW5zIGluIHNveWJlYW4geWllbGQgYnkgYSBxdWFydGVyLiBBZ3JpY3VsdHVyYWwgcHJhY3RpY2VzIEJpb2RpdmVyc2l0eSBhbmQgaGVhbHRoeSBzb2lsIGFyZSBjZW50cmFsIHRvIGVjb2xvZ2ljYWwgYXBwcm9hY2hlcyBmb3IgbW9yZSBkcm91Z2h0LXJlc2lzdGFudCBhbmQgZHJvdWdodC10b2xlcmFudCBmYXJtaW5nIHByYWN0aWNlcywgd2hpY2ggaXMgIG1vcmUgcmVzaWxpZW50IHRvIGV4dHJlbWUgZXZlbnRzLjI2IFByYWN0aWNlcyBlbmNvdXJhZ2luZyBiaW9kaXZlcnNpdHkgIGFuZCBzb2lsIGhlYWx0aCB3b3VsZCBpbmNyZWFzZSB0aGUgYXZhaWxhYmxlIHdhdGVyIGZvciBjcm9wcyBhbmQgaW1wcm92ZSAgc3VzdGFpbmFiaWxpdHkgb2YgYWdyaWN1bHR1cmFsIHByb2R1Y3Rpb24uMjcgTWFueSBvZiB0aGVzZSBhZ3JpY3VsdHVyYWwgcHJhY3RpY2VzIGFyZSBhdmFpbGFibGUgYW5kIGFyZSB1c2VkIGluIGRpZmZlcmVudCBwbGFjZXMgaW4gdGhlIHdvcmxkIHRvIG1hbmFnZSBjcm9wcyBhbmQgc29pbHMuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDcuMzkwMTYzNDIxNjMwODYsImxlZnQiOjkuNTIzODAwODQ5OTE0NTUsIndpZHRoIjozOS4xNjYyOTAyODMyMDMxMjUsImhlaWdodCI6MS4yMTAyNzc5MTUwMDA5MTU1fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDguNjEyNjEzNjc3OTc4NTE2LCJsZWZ0Ijo5LjUyMzgwMDg0OTkxNDU1LCJ3aWR0aCI6MzkuMTYyODcyMzE0NDUzMTI1LCJoZWlnaHQiOjEuMjEwMjc3OTE1MDAwOTE1NX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQ5LjgzNTA1MjQ5MDIzNDM3NSwibGVmdCI6OS41MjM4MDA4NDk5MTQ1NSwid2lkdGgiOjM5LjE2NjE0NTMyNDcwNzAzLCJoZWlnaHQiOjEuMjEwMjc3OTE1MDAwOTE1NX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjUxLjA1NzQ5NTExNzE4NzUsImxlZnQiOjkuNTIzODAwODQ5OTE0NTUsIndpZHRoIjoxMy40MTU0MjYyNTQyNzI0NjEsImhlaWdodCI6MS4yMTAyNzc5MTUwMDA5MTU1fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTIuOTUzMzMwOTkzNjUyMzQ0LCJsZWZ0Ijo5LjUyMzgwMDg0OTkxNDU1LCJ3aWR0aCI6MzkuMTY4MjgxNTU1MTc1NzgsImhlaWdodCI6MTMuNDM0NzE1MjcwOTk2MDk0fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NjcuMTk1OTQ1NzM5NzQ2MSwibGVmdCI6OS41MjM4MDA4NDk5MTQ1NSwid2lkdGgiOjE3LjE1Njk2NTI1NTczNzMwNSwiaGVpZ2h0IjoxLjcxMDAxOTU4ODQ3MDQ1OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjY5LjE1NTM5NTUwNzgxMjUsImxlZnQiOjkuNTIzODAwODQ5OTE0NTUsIndpZHRoIjozOS4xNjczMDExNzc5Nzg1MTYsImhlaWdodCI6Ni4xMDAwNDk5NzI1MzQxOH19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjc1Ljk0MTAwMTg5MjA4OTg0LCJsZWZ0Ijo5LjUyMzgwMDg0OTkxNDU1LCJ3aWR0aCI6MzkuMTYyMTQ3NTIxOTcyNjU2LCJoZWlnaHQiOjEuMjEwMjc3OTE1MDAwOTE1NX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjc3LjE2MzQ1OTc3NzgzMjAzLCJsZWZ0Ijo5LjUyMzgwMDg0OTkxNDU1LCJ3aWR0aCI6MzkuMTYyNzE1OTExODY1MjM0LCJoZWlnaHQiOjEuMjEwMjc3OTE1MDAwOTE1NX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjUifX0seyJvcmlnaW4iOjQsIndvcmsiOnsidHlwZSI6MzAsInN1YnR5cGUiOnt9LCJjb250cmlidXRvcnMiOlt7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJRaWFuZyIsImZhbWlseSI6IkNoYWkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiWWFudGFpIiwiZmFtaWx5IjoiR2FuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkNhaSIsImZhbWlseSI6IlpoYW8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSHVpLWxpYW4iLCJmYW1pbHkiOiJYdSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJSZWFnYW4iLCJmYW1pbHkiOiJXYXNrb20ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiWWluaW5nIiwiZmFtaWx5IjoiTml1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkthZGFtYm90IEguIE0uIiwiZmFtaWx5IjoiU2lkZGlxdWUifX1dLCJpZHMiOnsiaXNzbiI6WyIxNzczLTAxNTUiLCIxNzc0LTA3NDYiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMDcvczEzNTkzLTAxNS0wMzM4LTYiLCJtYWciOiIyMjIzOTYxMjgzIiwib3BlbmFsZXgiOiJXMjIyMzk2MTI4MyJ9LCJ0aXRsZSI6IlJlZ3VsYXRlZCBkZWZpY2l0IGlycmlnYXRpb24gZm9yIGNyb3AgcHJvZHVjdGlvbiB1bmRlciBkcm91Z2h0IHN0cmVzcy4gQSByZXZpZXciLCJhYnN0cmFjdCI6IkFncmljdWx0dXJlIGNvbnN1bWVzIG1vcmUgdGhhbiB0d28gdGhpcmRzIG9mIHRoZSB0b3RhbCBmcmVzaHdhdGVyIG9mIHRoZSBwbGFuZXQuIFRoaXMgaXNzdWUgY2F1c2VzIHN1YnN0YW50aWFsIGNvbmZsaWN0IGluIGZyZXNod2F0ZXIgYWxsb2NhdGlvbiBiZXR3ZWVuIGFncmljdWx0dXJlIGFuZCBvdGhlciBlY29ub21pYyBzZWN0b3JzLiBSZWd1bGF0ZWQgZGVmaWNpdCBpcnJpZ2F0aW9uIChSREkpIGlzIGtleSB0ZWNobm9sb2d5IGJlY2F1c2UgaXQgaGVscHMgdG8gaW1wcm92ZSB3YXRlciB1c2UgZWZmaWNpZW5jeS4gTm9uZXRoZWxlc3MsIHRoZXJlIGlzIGEgbGFjayBvZiB1bmRlcnN0YW5kaW5nIG9mIHRoZSBtZWNoYW5pc21zIHdpdGggd2hpY2ggcGxhbnRzIHJlc3BvbmQgdG8gUkRJLiBJbiBwYXJ0aWN1bGFyLCBsaXR0bGUgaXMga25vd24gYWJvdXQgaG93IFJESSBtaWdodCBpbmNyZWFzZSBjcm9wIHByb2R1Y3Rpb24gd2hpbGUgcmVkdWNpbmcgdGhlIGFtb3VudCBvZiBpcnJpZ2F0aW9uIHdhdGVyIGluIHJlYWwtd29ybGQgYWdyaWN1bHR1cmUuIEluIHRoaXMgcmV2aWV3LCB3ZSBmb3VuZCB0aGF0IFJESSBpcyBsYXJnZWx5IGltcGxlbWVudGVkIHRocm91Z2ggdGhyZWUgYXBwcm9hY2hlczogKDEpIGdyb3d0aCBzdGFnZS1iYXNlZCBkZWZpY2l0IGlycmlnYXRpb24sICgyKSBwYXJ0aWFsIHJvb3Qtem9uZSBpcnJpZ2F0aW9uLCBhbmQgKDMpIHN1YnN1cmZhY2UgZHJpcHBlciBpcnJpZ2F0aW9uLiBBbW9uZyB0aGVzZSwgcGFydGlhbCByb290LXpvbmUgaXJyaWdhdGlvbiBpcyB0aGUgbW9zdCBwb3B1bGFyIGFuZCBlZmZlY3RpdmUgYmVjYXVzZSBtYW55IGZpZWxkIGNyb3BzIGFuZCBzb21lIHdvb2R5IGNyb3BzIGNhbiBzYXZlIGlycmlnYXRpb24gd2F0ZXIgdXAgdG8gMjAgdG8gMzAgJSB3aXRob3V0IG9yIHdpdGggYSBtaW5pbWFsIGltcGFjdCBvbiBjcm9wIHlpZWxkLiBJbXByb3ZlZCB3YXRlciB1c2UgZWZmaWNpZW5jeSB3aXRoIFJESSBpcyBtYWlubHkgZHVlIHRvIHRoZSBmb2xsb3dpbmc6ICgxKSBlbmhhbmNlZCBndWFyZCBjZWxsIHNpZ25hbCB0cmFuc2R1Y3Rpb24gbmV0d29yayB0aGF0IGRlY3JlYXNlcyB0cmFuc3BpcmF0aW9uIHdhdGVyIGxvc3MsICgyKSBvcHRpbWl6ZWQgc3RvbWF0YWwgY29udHJvbCB0aGF0IGltcHJvdmVzIHRoZSBwaG90b3N5bnRoZXNpcyB0byB0cmFuc3BpcmF0aW9uIHJhdGlvLCBhbmQgKDMpIGRlY3JlYXNlZCBldmFwb3JhdGl2ZSBzdXJmYWNlIGFyZWFzIHdpdGggcGFydGlhbCByb290LXpvbmUgaXJyaWdhdGlvbiB0aGF0IHJlZHVjZXMgc29pbCBldmFwb3JhdGlvbi4gVGhlIG1lY2hhbmlzbXMgaW52b2x2ZWQgaW4gdGhlIHBsYW50IHJlc3BvbnNlIHRvIFJESS1pbmR1Y2VkIHdhdGVyIHN0cmVzcyBpbmNsdWRlIHRoZSBtb3JwaG9sb2dpY2FsIHRyYWl0cywgZS5nLiwgaW5jcmVhc2VkIHJvb3QgdG8gc2hvb3QgcmF0aW8gYW5kIGltcHJvdmVkIG51dHJpZW50IHVwdGFrZSBhbmQgcmVjb3Zlcnk7IHBoeXNpb2xvZ2ljYWwgdHJhaXRzLCBlLmcuLCBzdG9tYXRhbCBjbG9zdXJlLCBkZWNyZWFzZWQgbGVhZiByZXNwaXJhdGlvbiwgYW5kIG1haW50YWluZWQgcGhvdG9zeW50aGVzaXM7IGFuZCBiaW9jaGVtaWNhbCB0cmFpdHMsIGUuZy4sIGluY3JlYXNlZCBzaWduYWxpbmcgbW9sZWN1bGVzIGFuZCBlbmhhbmNlZCBhbnRpb3hpZGF0aW9uIGVuenltYXRpYyBhY3Rpdml0eS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAwNy9zMTM1OTMtMDE1LTAzMzgtNiIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9saW5rLnNwcmluZ2VyLmNvbS9jb250ZW50L3BkZi8xMC4xMDA3L3MxMzU5My0wMTUtMDMzOC02LnBkZiIsImlzc3VlZERhdGUiOiIyMDE1LTEyLTE4IiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiQWdyb25vbXkgZm9yIFN1c3RhaW5hYmxlIERldmVsb3BtZW50IiwiaXNzdWUiOjEsInZvbHVtZSI6MzYsImltcGFjdEZhY3RvciI6IjkuNzYifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiU3ByaW5nZXIgU2NpZW5jZStCdXNpbmVzcyBNZWRpYSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MTQ4LCJjaXRlZEJ5Q291bnQiOjU1M319LCJxdW90ZSI6IkFkZGl0aW9uYWxseSwgdGhlIHJlYWxsb2NhdGlvbiBvZiB3YXRlciByZXNvdXJjZXMgYW5kIG1vZGlmaWNhdGlvbiBvZiBpcnJpZ2F0aW9uIHN5c3RlbXMgbWF5IGhhdmUgYSByb2xlIHRvIHBsYXkgZm9yIHRoZSBpbXByb3ZlbWVudCBvZiB3YXRlciB1c2UgaW4gYWdyaWN1bHR1cmUgKEhvbWF5b3VuZmFyIGV0IGFsLiAyMDE0KS4gNy4yIENoYWxsZW5nZXMgVGhlIGNlbnRyYWwgbm92ZWx0eSBvZiBSREkgaXMgdGhhdCBpdCBhbGxvd3MgcGxhbnRzIHRvIGdyb3cgdW5kZXIgbWlsZCB3YXRlciBzdHJlc3MgdG8gaW5kdWNlIHJvb3Qtc291cmNlZCBjaGVtaWNhbCBzaWctbmFscyBmcm9tIGRyeWluZyByb290cyB0byByZWFjaCB0aGUgc2hvb3Qgd2hlcmUgcGh5c2lvbG9naWNhbCBhbmQgYmlvY2hlbWljYWwgcHJvY2Vzc2VzIGNhbiBiZSByZWd1bGF0ZWQuIEhvd2V2ZXIsIHRoZSBwaHlzaW9sb2dpY2FsIGFuZCBiaW9jaGVtaWNhbCByZXNwb25zZXMgYXJlIGRpZmZpY3VsdCB0byBxdWFuLXRpZnksIGdpdmVuIHRoYXQgcGFydGlhbCByb290LXpvbmUgaXJyaWdhdGlvbiBpcyB1c3VhbGx5IGFwcGxpZWQgdG8gcGxhbnRzIGluIGFjY29yZGFuY2Ugd2l0aCBib3RoIHRlbXBvcmFsIGFuZCBzcGF0aWFsIGRyb3VnaHRzLiBJbiBwcmFjdGljZSwgYWRvcHRpb24gb2Ygd2F0ZXItc2F2aW5nIHRlY2huaXF1ZSByZXF1aXJlcyBzY2ktZW50aWZpYyB1bmRlcnN0YW5kaW5nIG9mIHRoZSBtZWNoYW5pc21zIGludm9sdmVkIGluIHRoZSBwaHlzaW9sb2dpY2FsIGFuZCBiaW9jaGVtaWNhbCBwcm9jZXNzZXMuIEFsc28sIHRoZSBvdXRjb21lIG9mIHRoZSB3YXRlciBzdHJlc3MtaW5kdWNlZCByZXNwb25zZXMgaXMgaW5mbHVlbmNlZCBieSBtYW55IGZhY3RvcnMsIHN1Y2ggYXMgdGhlIHN0YWdlcyBvZiBjcm9wIGdyb3d0aCB3aGVuIHRoZSBkZWZpY2l0IGlzIGFwcGxpZWQsIHRoZSBpbnRlbnNpdHkgYW5kIHNldmVyaXR5IG9mIGRlZmljaXQgaW1wb3NlZCwgdGhlIG1ldGhvZCB3aXRoIHdoaWNoIGRlZmljaXQgaXJyaWdhdGlvbiBpcyBhcHBsaWVkLCBjcm9wIHNwZWNpZXMgYW5kIGN1bHRpdmFycywgYW5kIHRoZWlyIHJlc3BlY3RpdmUgbm9uLWNyaXRpY2FsIGdyb3d0aCBzdGFnZXMgd2hlbiB0aGUgc3RhZ2UtYmFzZWQgZGVmaWNpdCBpcnJpZ2F0aW9uIG1heSBiZSBhcHBsaWVkLiBUaGVzZSBmYWN0b3JzIG5lZWQgdG8gYmUgaW52ZXN0aWdhdGVkIGF0IGEgc3lzdGVtIGxldmVsIHVuZGVyIHNwZWNpZmljIGdyb3dpbmcgY29uZGl0aW9ucy4gUkRJIGFuZCwgcGFydGljdWxhcmx5LCBhbHRlcm5hdGUgcGFydGlhbCByb290LXpvbmUgaXJyaWdhdGlvbiBhcmUgc3RpbGwgcmVsYXRpdmVseSBuZXcgdG8gbWFuYWdlIGZhcm0gbWFuYWdlcnMuIFRoaXMgdGVjaC1uaXF1ZSBpcyBub3QgZWFzeSB0byBpbXBsZW1lbnQgaW4gY3JvcCBzcGVjaWVzIHdpdGggZGVuc2UgcGxhbnQgcG9wdWxhdGlvbiBzdWNoIGFzIGNlcmVhbCBhbmQgb2lsc2VlZCBjcm9wcy4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEzLCJiYm94Ijp7InRvcCI6NjUuNTQxNDgxMDE4MDY2NCwibGVmdCI6NTEuNDMxODQ2NjE4NjUyMzQ0LCJ3aWR0aCI6MzkuOTk0MjgxNzY4Nzk4ODMsImhlaWdodCI6MS4zNjQyNjI3MDAwODA4NzE2fX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjY3LjExODY0NDcxNDM1NTQ3LCJsZWZ0Ijo1MS40MzE4NDY2MTg2NTIzNDQsIndpZHRoIjozOS45OTU5NDExNjIxMDkzNzUsImhlaWdodCI6MS4zNjQyNjI3MDAwODA4NzE2fX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjY4LjY4ODI0MDA1MTI2OTUzLCJsZWZ0Ijo1MS40MzE4NDY2MTg2NTIzNDQsIndpZHRoIjozOS45OTI1ODA0MTM4MTgzNiwiaGVpZ2h0IjoxLjM2NDI2MjcwMDA4MDg3MTZ9fSx7InBhZ2VOdW1iZXIiOjEzLCJiYm94Ijp7InRvcCI6NzAuMjY1Mzk2MTE4MTY0MDYsImxlZnQiOjUxLjQzMTg0NjYxODY1MjM0NCwid2lkdGgiOjE3Ljg5MDg5MjAyODgwODU5NCwiaGVpZ2h0IjoxLjM2NDI2MjcwMDA4MDg3MTZ9fSx7InBhZ2VOdW1iZXIiOjEzLCJiYm94Ijp7InRvcCI6NzMuNDA3MTA0NDkyMTg3NSwibGVmdCI6NTEuNDMxODQ2NjE4NjUyMzQ0LCJ3aWR0aCI6MTAuMTAzNTkxOTE4OTQ1MzEyLCJoZWlnaHQiOjEuMzc2ODU3NjM4MzU5MDY5OH19LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjo3Ni41NjY0NTk2NTU3NjE3MiwibGVmdCI6NTEuNDMxODQ2NjE4NjUyMzQ0LCJ3aWR0aCI6NDAuMDA3NjcxMzU2MjAxMTcsImhlaWdodCI6MTMuOTU4ODIwMzQzMDE3NTc4fX0seyJwYWdlTnVtYmVyIjoxNCwiYmJveCI6eyJ0b3AiOjcuMjg0ODU4NzAzNjEzMjgxLCJsZWZ0Ijo4LjU3MTQxODc2MjIwNzAzMSwid2lkdGgiOjQwLjAwNzY0MDgzODYyMzA1LCJoZWlnaHQiOjEuMzY0MjYzMTc2OTE4MDI5OH19LHsicGFnZU51bWJlciI6MTQsImJib3giOnsidG9wIjo4Ljg1NDQ1MzA4Njg1MzAyNywibGVmdCI6OC41NzE0MTg3NjIyMDcwMzEsIndpZHRoIjo0MC4wMTU5OTUwMjU2MzQ3NjYsImhlaWdodCI6MS4zNjQyNjM2NTM3NTUxODh9fSx7InBhZ2VOdW1iZXIiOjE0LCJiYm94Ijp7InRvcCI6MTAuNDMxNjExMDYxMDk2MTkxLCJsZWZ0Ijo4LjU3MTQxODc2MjIwNzAzMSwid2lkdGgiOjQwLjAwNzY0MDgzODYyMzA1LCJoZWlnaHQiOjEuMzY0MjYzNjUzNzU1MTg4fX0seyJwYWdlTnVtYmVyIjoxNCwiYmJveCI6eyJ0b3AiOjEyLjAwODc2MTQwNTk0NDgyNCwibGVmdCI6OC41NzE0MTg3NjIyMDcwMzEsIndpZHRoIjo0MC4wMDQzMDI5Nzg1MTU2MjUsImhlaWdodCI6MS4zNjQyNjM2NTM3NTUxODh9fSx7InBhZ2VOdW1iZXIiOjE0LCJiYm94Ijp7InRvcCI6MTMuNTc4MzU1Nzg5MTg0NTcsImxlZnQiOjguNTcxNDE4NzYyMjA3MDMxLCJ3aWR0aCI6MzkuOTk3NTk2NzQwNzIyNjU2LCJoZWlnaHQiOjEuMzY0MjYzNjUzNzU1MTg4fX0seyJwYWdlTnVtYmVyIjoxNCwiYmJveCI6eyJ0b3AiOjE1LjE1NTUxMzc2MzQyNzczNCwibGVmdCI6OC41NzE0MTg3NjIyMDcwMzEsIndpZHRoIjo0MC4wMTA5OTM5NTc1MTk1MywiaGVpZ2h0IjoxLjM2NDI2MzY1Mzc1NTE4OH19LHsicGFnZU51bWJlciI6MTQsImJib3giOnsidG9wIjoxNi43MzI2NjQxMDgyNzYzNjcsImxlZnQiOjguNTcxNDE4NzYyMjA3MDMxLCJ3aWR0aCI6NDAuMDEwOTkwMTQyODIyMjY2LCJoZWlnaHQiOjEuMzY0MjYyNzAwMDgwODcxNn19LHsicGFnZU51bWJlciI6MTQsImJib3giOnsidG9wIjoxOC4zMDIyNTk0NDUxOTA0MywibGVmdCI6OC41NzE0MTg3NjIyMDcwMzEsIndpZHRoIjo0MC4wMDc2MzcwMjM5MjU3OCwiaGVpZ2h0IjoxLjM2NDI2MjcwMDA4MDg3MTZ9fSx7InBhZ2VOdW1iZXIiOjE0LCJiYm94Ijp7InRvcCI6MTkuODc5NDE3NDE5NDMzNTk0LCJsZWZ0Ijo4LjU3MTQxODc2MjIwNzAzMSwid2lkdGgiOjEzLjA4OTMwNDkyNDAxMTIzLCJoZWlnaHQiOjEuMzY0MjYyNzAwMDgwODcxNn19LHsicGFnZU51bWJlciI6MTQsImJib3giOnsidG9wIjoyMS40NTY1Njc3NjQyODIyMjcsImxlZnQiOjEwLjQ3NTk4NzQzNDM4NzIwNywid2lkdGgiOjM4LjExMTQyNzMwNzEyODkwNiwiaGVpZ2h0IjoxLjM2NDI2MjcwMDA4MDg3MTZ9fSx7InBhZ2VOdW1iZXIiOjE0LCJiYm94Ijp7InRvcCI6MjMuMDI2MTYzMTAxMTk2MjksImxlZnQiOjguNTcxNDE4NzYyMjA3MDMxLCJ3aWR0aCI6MzkuOTkyNTg4MDQzMjEyODksImhlaWdodCI6MS4zNjQyNjI3MDAwODA4NzE2fX0seyJwYWdlTnVtYmVyIjoxNCwiYmJveCI6eyJ0b3AiOjI0LjYwMzMyMTA3NTQzOTQ1MywibGVmdCI6OC41NzE0MTg3NjIyMDcwMzEsIndpZHRoIjo0MC4wMDI2MTMwNjc2MjY5NSwiaGVpZ2h0IjoxLjM2NDI2MjcwMDA4MDg3MTZ9fSx7InBhZ2VOdW1iZXIiOjE0LCJiYm94Ijp7InRvcCI6MjYuMTgwNDY5NTEyOTM5NDUzLCJsZWZ0Ijo4LjU3MTQxODc2MjIwNzAzMSwid2lkdGgiOjM5Ljk5NTkyMjA4ODYyMzA1LCJoZWlnaHQiOjEuMzY0MjYyNzAwMDgwODcxNn19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjE0In19XQ%3D%3D. Emerging technologies like AI-driven smart irrigation, synthetic biology for microbial consortia, and OpenCRISPR democratize tools for resilience, promising optimized WUE via real-time soil data (Xie, 2025)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjMwLCJzdWJ0eXBlIjp7fSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQ2hlbiIsImZhbWlseSI6IlhpZSJ9fV0sImlkcyI6eyJpc3NuIjpbIjE2NjQtNDYyWCJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMzM4OS9mcGxzLjIwMjUuMTY3NTY1NyIsInBtaWQiOiI0MTAzNjM5MyIsIm9wZW5hbGV4IjoiVzQ0MTQyNDgzMDcifSwidGl0bGUiOiJUaGUgcm9sZSBvZiBtb2Rlcm4gYWdyaWN1bHR1cmFsIHRlY2hub2xvZ2llcyBpbiBpbXByb3ZpbmcgYWdyaWN1bHR1cmFsIHByb2R1Y3Rpdml0eSBhbmQgbGFuZCB1c2UgZWZmaWNpZW5jeSIsImFic3RyYWN0IjoiTW9kZXJuIGFncmljdWx0dXJhbCB0ZWNobm9sb2dpZXMgYXJlIGNydWNpYWwgZm9yIGFkZHJlc3NpbmcgZ2xvYmFsIGZvb2Qgc2VjdXJpdHkgYW5kIGVudmlyb25tZW50YWwgc3VzdGFpbmFiaWxpdHkgY2hhbGxlbmdlcyBhbWlkc3QgYSBncm93aW5nIHBvcHVsYXRpb24gYW5kIGNsaW1hdGUgY2hhbmdlLiBUaGVzZSBpbm5vdmF0aW9ucywgaW5jbHVkaW5nIHByZWNpc2lvbiBhZ3JpY3VsdHVyZSwgYmlvdGVjaG5vbG9neSwgc21hcnQgaXJyaWdhdGlvbiwgYXV0b21hdGlvbiwgdmVydGljYWwgZmFybWluZywgYW5kIGFydGlmaWNpYWwgaW50ZWxsaWdlbmNlIChBSSksIHNpZ25pZmljYW50bHkgZW5oYW5jZSBwcm9kdWN0aXZpdHkgYW5kIGxhbmQgdXNlIGVmZmljaWVuY3kuIFByZWNpc2lvbiBhZ3JpY3VsdHVyZSwgdXRpbGl6aW5nIEdQUywgZHJvbmVzLCBhbmQgSW9ULCBpbXByb3ZlcyB5aWVsZHMgYnkgMjDigJMzMCUgYW5kIGN1dHMgaW5wdXQgd2FzdGUgYnkgNDDigJM2MCUuIEJpb3RlY2hub2xvZ3ksIHdpdGggQ1JJU1BSIGFuZCBHTU9zLCBkZWxpdmVycyBkcm91Z2h0IGFuZCBwZXN0LXJlc2lzdGFudCBjcm9wcywgc3RhYmlsaXppbmcgeWllbGRzLCBhcyBzZWVuIHdpdGggQnQgY290dG9uIHJlZHVjaW5nIHBlc3RpY2lkZSB1c2UgYnkgNTAlIGluIEluZGlhLiBTbWFydCBpcnJpZ2F0aW9uIGJvb3N0cyB3YXRlciBlZmZpY2llbmN5IGJ5IDQw4oCTNjAlLCB3aGlsZSBhdXRvbWF0aW9uIGFuZCByb2JvdGljcyBtaXRpZ2F0ZSBsYWJvciBzaG9ydGFnZXMgYW5kIHJlZHVjZSBjb3N0cyBieSAyNSUuIFZlcnRpY2FsIGZhcm1pbmcgaW5jcmVhc2VzIHlpZWxkcyAxMOKAkzIwIHRpbWVzIHdpdGggOTUlIGxlc3MgbGFuZCBhbmQgd2F0ZXIsIHN1cHBvcnRpbmcgdXJiYW4gZm9vZCBzZWN1cml0eS4gQUkgYW5hbHl0aWNzIGVuaGFuY2UgZGVjaXNpb24tbWFraW5nIHdpdGggb3ZlciA5MCUgYWNjdXJhY3kgaW4gZm9yZWNhc3RpbmcgYW5kIHJlc291cmNlIGFsbG9jYXRpb24uIERlc3BpdGUgdGhlc2UgYmVuZWZpdHMsIGhpZ2ggY29zdHMsIHRlY2hub2xvZ2ljYWwgaWxsaXRlcmFjeSwgYW5kIHJlZ3VsYXRvcnkgaXNzdWVzIGhpbmRlciBhZG9wdGlvbiwgZXNwZWNpYWxseSBhbW9uZyBzbWFsbGhvbGRlcnMuIFBvbGljeSBzdXBwb3J0LCBwdWJsaWMtcHJpdmF0ZSBwYXJ0bmVyc2hpcHMsIGFuZCB0cmFpbmluZyBhcmUgdml0YWwgZm9yIGJyb2FkZXIgdGVjaG5vbG9neSBhY2Nlc3MgYW5kIGZhaXIgYmVuZWZpdHMuIEludGVncmF0aW5nIHJlbmV3YWJsZSBlbmVyZ3kgYW5kIGNpcmN1bGFyIGVjb25vbXkgcHJpbmNpcGxlcyBpbnRvIGFnZ3JvdGVjaCBwcmVzZW50cyBhIHBhdGggdG8gc3VzdGFpbmFiaWxpdHkuIFRoaXMgcmV2aWV3IGhpZ2hsaWdodHMgdGhlIHRyYW5zZm9ybWF0aXZlIHBvdGVudGlhbCBvZiBtb2Rlcm4gdGVjaG5vbG9naWVzIGZvciBzdXN0YWluYWJsZSBpbnRlbnNpZmljYXRpb24sIGluY3JlYXNpbmcgcHJvZHVjdGl2aXR5IHdpdGhvdXQgZXhwYW5kaW5nIGZhcm1sYW5kLCB3aGlsZSBsZXNzZW5pbmcgZW52aXJvbm1lbnRhbCBpbXBhY3RzLiBJdCB1bmRlcnNjb3JlcyB0aGUgbmVlZCBmb3IgY29vcmRpbmF0ZWQgZWZmb3J0cyB0byBvdmVyY29tZSBhZG9wdGlvbiBjaGFsbGVuZ2VzIGFuZCBoYXJuZXNzIHRoZXNlIGlubm92YXRpb25zIGZvciBnbG9iYWwgZm9vZCBzZWN1cml0eSBhbmQgY2xpbWF0ZSByZXNpbGllbmNlLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzg5L2ZwbHMuMjAyNS4xNjc1NjU3IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5mcm9udGllcnNpbi5vcmcvam91cm5hbHMvcGxhbnQtc2NpZW5jZS9hcnRpY2xlcy8xMC4zMzg5L2ZwbHMuMjAyNS4xNjc1NjU3L3BkZiIsImlzc3VlZERhdGUiOiIyMDI1LTA5LTE2IiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiRnJvbnRpZXJzIGluIFBsYW50IFNjaWVuY2UiLCJ2b2x1bWUiOjE2LCJpbXBhY3RGYWN0b3IiOiI3LjAwIn0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkZyb250aWVycyBNZWRpYSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6ODgsImNpdGVkQnlDb3VudCI6MH19LCJxdW90ZSI6IlN5bnRoZXRpYyBiaW9sb2d5IGlzIGFsc28gY29udHJpYnV0aW5nLCB3aXRoIG1pY3JvYmlhbCBjb25zb3J0aWEgZW5naW5lZXJlZCB0byBlbmhhbmNlIHNvaWwgaGVhbHRoIGFuZCBjcm9wIHJlc2lsaWVuY2UgKExhd3NvbiBldCBhbC4sIDIwMTkpLiBEZXNwaXRlIHRoZXNlIGFkdmFuY2VzLCBlcXVpdGFibGUgYWNjZXNzIHJlbWFpbnMgYSBjaGFsbGVuZ2UsIGFzIHNtYWxsaG9sZGVyIGZhcm1lcnMgaW4gZGV2ZWxvcGluZyByZWdpb25zIG9mdGVuIGxhY2sgYWNjZXNzIHRvIHBhdGVudGVkIHRlY2hub2xvZ2llcyAoVG9lbm5pZXNzZW4gZXQgYWwuLCAyMDA4KS4gSW5pdGlhdGl2ZXMgbGlrZSB0aGUgT3BlbkNSSVNQUiBwcm9qZWN0IGFpbSB0byBkZW1vY3JhdGl6ZSBnZW5lLWVkaXRpbmcgdG9vbHMgKENvaGVuLCAyMDIzKSwgd2hpbGUgcHVibGljLXNlY3RvciBwYXJ0bmVyc2hpcHMgKGUuZy4sIENJTU1ZVOKAmXMgZHJvdWdodC10b2xlcmFudCBtYWl6ZSkgZW5zdXJlIGFmZm9yZGFiaWxpdHkuIEFzIGNsaW1hdGUgY2hhbmdlIGludGVuc2lmaWVzLCB0aGVzZSBiaW90ZWNobm9sb2dpZXMgd2lsbCBiZSBpbmRpc3BlbnNhYmxlIGZvciBhY2hpZXZpbmcgc3VzdGFpbmFibGUgaW50ZW5zaWZpY2F0aW9uLCBidXQgdGhlaXIgc3VjY2VzcyBoaW5nZXMgb24gZ2xvYmFsIGNvbGxhYm9yYXRpb24sIGV0aGljYWwgZ292ZXJuYW5jZSwgYW5kIGluY2x1c2l2ZSBpbm5vdmF0aW9uLiAzLjMgU21hcnQgaXJyaWdhdGlvbiAmIHdhdGVyIG1hbmFnZW1lbnQgV2F0ZXIgc2NhcmNpdHkgcG9zZXMgb25lIG9mIHRoZSBtb3N0IHNpZ25pZmljYW50IGNoYWxsZW5nZXMgdG8gZ2xvYmFsIGFncmljdWx0dXJlLCB3aXRoIDcwJSBvZiBmcmVzaHdhdGVyIHdpdGhkcmF3YWxzIGFsbG9jYXRlZCB0byBpcnJpZ2F0aW9uLCBvZnRlbiBpbmVmZmljaWVudGx5IChGQU8sIDIwMjApLiBTbWFydCBpcnJpZ2F0aW9uIHN5c3RlbXMsIGludGVncmF0aW5nIElvVC1iYXNlZCBzZW5zb3JzIGFuZCBkcmlwIGlycmlnYXRpb24gdGVjaG5vbG9naWVzLCBhcmUgdHJhbnNmb3JtaW5nIHdhdGVyIG1hbmFnZW1lbnQgYnkgb3B0aW1pemluZyB1c2FnZSwgcmVkdWNpbmcgd2FzdGUsIGFuZCBlbmhhbmNpbmcgY3JvcCB5aWVsZHMuIFNvaWwgbW9pc3R1cmUgc2Vuc29ycywgc3VjaCBhcyBjYXBhY2l0aXZlIHByb2JlcyBhbmQgdGVuc2lvbWV0ZXJzLCBwcm92aWRlIHJlYWwtdGltZSBkYXRhIG9uIHdhdGVyIGF2YWlsYWJpbGl0eSBhdCB2YXJ5aW5nIHJvb3QgZGVwdGhzLCBlbmFibGluZyBwcmVjaXNlIGlycmlnYXRpb24gc2NoZWR1bGluZy4gVGhlc2Ugc2Vuc29ycyB0cmFuc21pdCBkYXRhIHZpYSBMb1JhV0FOIG9yIGNlbGx1bGFyIG5ldHdvcmtzIHRvIGNsb3VkIHBsYXRmb3Jtcywgd2hlcmUgQUkgYWxnb3JpdGhtcyBhbmFseXplIHRyZW5kcyBhbmQgYXV0b21hdGUgaXJyaWdhdGlvbiBkZWNpc2lvbnMgKEdvYXAgZXQgYWwuLCAyMDE4KS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo1Ny4xNzU2NDM5MjA4OTg0NCwibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ5MzE3NTUwNjU5MTgsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NTguNjY0MjM0MTYxMzc2OTUsImxlZnQiOjguMzMxNDQyODMyOTQ2Nzc3LCJ3aWR0aCI6NDAuNDkzOTUzNzA0ODMzOTg0LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjYwLjE0NjA4MDAxNzA4OTg0NCwibGVmdCI6OC4zMzE0NDE4NzkyNzI0NjEsIndpZHRoIjo0MC40OTc1ODE0ODE5MzM1OTQsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NjEuNjM0Njc3ODg2OTYyODksImxlZnQiOjguMzMxNDQxODc5MjcyNDYxLCJ3aWR0aCI6NDAuNDk1OTA2ODI5ODMzOTg0LCJoZWlnaHQiOjEuMTgwNTE2OTU4MjM2Njk0M319LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjYzLjExNjUyNzU1NzM3MzA1LCJsZWZ0Ijo4LjMzMTQ0MjgzMjk0Njc3Nywid2lkdGgiOjQwLjQ5NzU3NzY2NzIzNjMzLCJoZWlnaHQiOjEuMTgwNTE2OTU4MjM2Njk0M319LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjY0LjYwNTExNzc5Nzg1MTU2LCJsZWZ0Ijo4LjMzMTQzOTk3MTkyMzgyOCwid2lkdGgiOjQwLjQ4ODM0NjA5OTg1MzUxNiwiaGVpZ2h0IjoxLjE4MDUxNjk1ODIzNjY5NDN9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo2Ni4wODY5Njc0NjgyNjE3MiwibGVmdCI6OC4zMzE0NDE4NzkyNzI0NjEsIndpZHRoIjo0MC40ODMyMDc3MDI2MzY3MiwiaGVpZ2h0IjoxLjE4MDUxNjk1ODIzNjY5NDN9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo2Ny41NzU1NTM4OTQwNDI5NywibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ4MzA1MTMwMDA0ODgzLCJoZWlnaHQiOjEuMTgwNTE2OTU4MjM2Njk0M319LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjY5LjA1NzQwMzU2NDQ1MzEyLCJsZWZ0Ijo4LjMzMTU5MTYwNjE0MDEzNywid2lkdGgiOjQwLjQ4MTg2MTExNDUwMTk1LCJoZWlnaHQiOjEuMTgwNTE2OTU4MjM2Njk0M319LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjcwLjU0NTk5NzYxOTYyODksImxlZnQiOjguMzMxNTg5Njk4NzkxNTA0LCJ3aWR0aCI6NDAuNDg2MDgzOTg0Mzc1LCJoZWlnaHQiOjEuMTgwNTE2OTU4MjM2Njk0M319LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjcyLjAyNzg0NzI5MDAzOTA2LCJsZWZ0Ijo4LjMzMTQzOTAxODI0OTUxMiwid2lkdGgiOjM4LjIxMTg3MjEwMDgzMDA4LCJoZWlnaHQiOjEuMTgwNTE2OTU4MjM2Njk0M319LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjc2LjEzOTU0MTYyNTk3NjU2LCJsZWZ0Ijo4LjMzMTc0NzA1NTA1MzcxMSwid2lkdGgiOjM4LjQ1NjgyMTQ0MTY1MDM5LCJoZWlnaHQiOjEuNTU4MzMyMjA0ODE4NzI1Nn19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjc5LjQ1ODYyNTc5MzQ1NzAzLCJsZWZ0Ijo4LjMzMTU5OTIzNTUzNDY2OCwid2lkdGgiOjQwLjUwOTY1ODgxMzQ3NjU2LCJoZWlnaHQiOjExLjU3MzY4NjU5OTczMTQ0NX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjkuNjU1MTM2MTA4Mzk4NDM4LCJsZWZ0Ijo1MS4yMDkyMTcwNzE1MzMyLCJ3aWR0aCI6NDAuNDg1NjQ1Mjk0MTg5NDUsImhlaWdodCI6MS4xODA1MTk1ODA4NDEwNjQ1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6MTEuMTM2OTkxNTAwODU0NDkyLCJsZWZ0Ijo1MS4yMDkyMTcwNzE1MzMyLCJ3aWR0aCI6NDAuNDgzMTk2MjU4NTQ0OTIsImhlaWdodCI6MS4xODA1MTk1ODA4NDEwNjQ1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6MTIuNjI1NTgzNjQ4NjgxNjQsImxlZnQiOjUxLjIwOTIxNzA3MTUzMzIsIndpZHRoIjo0MC40OTA3NTY5ODg1MjUzOSwiaGVpZ2h0IjoxLjE4MDUxOTU4MDg0MTA2NDV9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI2In19XQ%3D%3D. Future research must prioritize smallholder-accessible innovations and genotype-environment interactions (Chai et al., 2015; Chami&Moujabber, 2016)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkRhbmllbCBFbCIsImZhbWlseSI6IkNoYW1pIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1hcm91biBFbCIsImZhbWlseSI6Ik1vdWphYmJlciJ9fV0sImlkcyI6eyJpc3NuIjpbIjAwMzgtMjM1MyIsIjE5OTYtNzQ4OSJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMTcxNTkvc2Fqcy4yMDE2LzIwMTUwNDU3IiwibWFnIjoiMjUyMzY3OTY5OCIsIm9wZW5hbGV4IjoiVzI1MjM2Nzk2OTgifSwidGl0bGUiOiJEcm91Z2h0LCBjbGltYXRlIGNoYW5nZSBhbmQgc3VzdGFpbmFiaWxpdHkgb2Ygd2F0ZXIgaW4gYWdyaWN1bHR1cmU6IEEgcm9hZG1hcCB0b3dhcmRzIHRoZSBOV1JTMiIsImFic3RyYWN0IjoiVGhlIGZyZXF1ZW5jeSBhbmQgaW50ZW5zaXR5IG9mIGRyb3VnaHQsIGV4dHJlbWUgZXZlbnRzIGFuZCBoaWdoIHdpbmQgdmVsb2NpdGllcyBpbiBTb3V0aCBBZnJpY2EgYXJlIGV4cGVjdGVkIHRvIGluY3JlYXNlIGluIHRoZSBuZXh0IGNlbnR1cnkgYXMgYSByZXN1bHQgb2YgdGhlIGNoYW5naW5nIGNsaW1hdGUuIFRoZSBOYXRpb25hbCBXYXRlciBSZXNvdXJjZSBTdHJhdGVneSAyIChOV1JTMikgaGFzIHNldCBvdXQgdGhlIGdlbmVyYWwgYW5kIHN0cmF0ZWdpYyBkaXJlY3Rpb25zIGZvciB3YXRlciByZXNvdXJjZXMgbWFuYWdlbWVudCBpbiB0aGUgY291bnRyeSBmb3IgdGhlIG5leHQgMjAgeWVhcnMuIEhvd2V2ZXIsIHRoZSBzdHJhdGVneSBkb2VzIG5vdCBkcmF3IGEgZnJhbWV3b3JrIHRhaWxvcmVkIHNwZWNpZmljYWxseSBmb3IgYWdyaWN1bHR1cmFsIHVzZSwgd2l0aCBzcGVjaWZpYyBtZWFzdXJlcyBhbmQgZ29hbHMuIFRoZXJlZm9yZSwgdG8gcmVhY2ggc3VzdGFpbmFiaWxpdHkgb2Ygd2F0ZXIgaW4gYWdyaWN1bHR1cmUsIGZvdXIgbWFqb3Igc3RyYXRlZ2ljIGdvYWxzIGFyZSBzdWdnZXN0ZWQsIG9uIHdoaWNoIHJlc2VhcmNoIGluc3RpdHV0aW9ucyBjYW4gZm9jdXMgYW5kIHByb21vdGUgdGhyb3VnaCBnb29kIGdvdmVybmFuY2UuIFRoZSBzdHJhdGVneSBlbXBoYXNpc2VzOiAoMSkgY3JvcCByZXNlYXJjaCB0byBmaW5kIG5ldyBkcm91Z2h0LWFuZCBoZWF0LSB0b2xlcmFudCBhbmQgcmVzaXN0YW50IGJyZWVkcyBhbmQgdmFyaWV0aWVzOyAoMikgaW50ZW5zaWZpZWQgcmVzZWFyY2ggaW4gYWdyaWN1bHR1cmFsIHByYWN0aWNlczsgKDMpIGluY3JlYXNpbmcgdGhlIGVmZmljaWVuY3kgb2Ygd2F0ZXIgdXNlIHdpdGhpbiBhZ3JpY3VsdHVyZTsgYW5kICg0KSBpbnRlZ3JhdGluZyBhbGwgdGhlc2Ugc3RyYXRlZ2ljIGdvYWxzIHdpdGhpbiBhIHN1c3RhaW5hYmxlIHJlc2VhcmNoIGZyYW1ld29yay4gRmluYWxseSwgdGhlIHJlc2VhcmNoIGNhbGxzIGZvciByYXBpZCBhY3Rpb24gYW5kIGltcGxlbWVudGF0aW9uLiBTaWduaWZpY2FuY2U6IFRoZSBmcmFtZXdvcmsgaXMgcHJvcG9zZWQgZm9yIHN0YWtlaG9sZGVycyBhbmQgcG9saWN5bWFrZXJzIGluIGhpZ2hlciBlZHVjYXRpb24sIGFncmljdWx0dXJlIGFuZCByZXNvdXJjZXMgbWFuYWdlbWVudCBpbiBTb3V0aCBBZnJpY2EgZm9yIG5ldyByZXNlYXJjaCBob3Jpem9ucyBhdCBuYXRpb25hbCBsZXZlbCB0byBpbXByb3ZlIG92ZXJhbGwgYWdyaWN1bHR1cmFsIHN1c3RhaW5hYmlsaXR5IGJ5IDIwMzAgYXMgc3RpcHVsYXRlZCBieSB0aGUgTWlsbGVubml1bSBEZXZlbG9wbWVudCBHb2Fscy4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTcxNTkvc2Fqcy4yMDE2LzIwMTUwNDU3IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5zYWpzLmNvLnphL2FydGljbGUvZG93bmxvYWQvMzY2My80NzY5IiwiaXNzdWVkRGF0ZSI6IjIwMTYtMDktMjMiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA5IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJTb3V0aCBBZnJpY2FuIEpvdXJuYWwgb2YgU2NpZW5jZSIsInZvbHVtZSI6MTEyLCJpbXBhY3RGYWN0b3IiOiIxLjQyIn0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6NCwibGFzdFBhZ2UiOjR9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiQWNhZGVteSBvZiBTY2llbmNlIG9mIFNvdXRoIEFmcmljYSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MzksImNpdGVkQnlDb3VudCI6MjV9fSwicXVvdGUiOiJJbiB0aGUgbWVhbnRpbWUsIGNyb3AgYnJlZWRpbmcgKGVzcGVjaWFsbHkgdGhlICBjb252ZW50aW9uYWwgYnJlZWRpbmcgdXNpbmcgbWFya2VyLWFzc2lzdGVkIHNlbGVjdGlvbikgaXMgYW4gZXh0cmVtZWx5ICB1c2VmdWwgdGVjaG5pcXVlIGZvciBicmVlZGluZyBjb21wbGV4IHRyYWl0cyAoc3VjaCBhcyBkcm91Z2h0IHRvbGVyYW5jZSkgIGludG8gaW1wcm92ZWQgdmFyaWV0aWVzLjIyIFRoZXJlIGlzIGEgZGVhcnRoIG9mIHN0dWRpZXMgcmVwb3J0ZWQgaW4gdGhlIGxpdGVyYXR1cmUgdG8gZW5hYmxlIHF1YW50aS1maWNhdGlvbiBvZiB3YXRlciBjb25zdW1wdGlvbiBhbmQgc2F2aW5ncyBhcyBhIHJlc3VsdCBvZiBzdWNoIGltcHJvdmVkIHZhcmlldGllcy4gSG93ZXZlciwgYSBzdHVkeSBoYXMgc2hvd24gdGhhdCBkcm91Z2h0LXRvbGVyYW50IGFuZCBkcm91Z2h0LXJlc2lzdGFudCB0cmFpdHMgY291bGQgaW1wcm92ZSB0aGUgeWllbGQgb2YgbWFpemUgaW4gQWZyaWNhIGJ5IHVwIHRvIDMwJSAgdW5kZXIgZHJvdWdodCBjb25kaXRpb25zIHdoZW4gY29tcGFyZWQgd2l0aCBjb21tZXJjaWFsIHNlZWRzLjIzICBGdXJ0aGVybW9yZSwgR3VyIGFuZCBaYW1pcjI0IHN1Y2Nlc3NmdWxseSBvYnRhaW5lZCA1MCUgaGlnaGVyIHlpZWxkcyBvZiAgdG9tYXRvLCBldmVuIHVuZGVyIGRyb3VnaHQgY29uZGl0aW9ucywgYnkgYnJlZWRpbmcgY3VsdGl2YXRlZCB0b21hdG9lcyAgd2l0aCB3aWxkIHJlbGF0aXZlcy4gRGVyeW5nIGV0IGFsLjI1IGZvdW5kIOKAkyBvbiBhIGdsb2JhbCBzY2FsZSDigJMgdGhhdCBieSB0aGUgIDIwODBzLCBleHRyZW1lIGhlYXQgc3RyZXNzIGF0IGFudGhlc2lzIGNvdWxkOiAoMSkgZG91YmxlIGxvc3NlcyBvZiBtYWl6ZSAgeWllbGQsICgyKSByZWR1Y2UgcHJvamVjdGVkIGdhaW5zIGluIHNwcmluZyB3aGVhdCB5aWVsZCBieSBoYWxmLCBhbmQgKDMpICByZWR1Y2UgcHJvamVjdGVkIGdhaW5zIGluIHNveWJlYW4geWllbGQgYnkgYSBxdWFydGVyLiBBZ3JpY3VsdHVyYWwgcHJhY3RpY2VzIEJpb2RpdmVyc2l0eSBhbmQgaGVhbHRoeSBzb2lsIGFyZSBjZW50cmFsIHRvIGVjb2xvZ2ljYWwgYXBwcm9hY2hlcyBmb3IgbW9yZSBkcm91Z2h0LXJlc2lzdGFudCBhbmQgZHJvdWdodC10b2xlcmFudCBmYXJtaW5nIHByYWN0aWNlcywgd2hpY2ggaXMgIG1vcmUgcmVzaWxpZW50IHRvIGV4dHJlbWUgZXZlbnRzLjI2IFByYWN0aWNlcyBlbmNvdXJhZ2luZyBiaW9kaXZlcnNpdHkgIGFuZCBzb2lsIGhlYWx0aCB3b3VsZCBpbmNyZWFzZSB0aGUgYXZhaWxhYmxlIHdhdGVyIGZvciBjcm9wcyBhbmQgaW1wcm92ZSAgc3VzdGFpbmFiaWxpdHkgb2YgYWdyaWN1bHR1cmFsIHByb2R1Y3Rpb24uMjcgTWFueSBvZiB0aGVzZSBhZ3JpY3VsdHVyYWwgcHJhY3RpY2VzIGFyZSBhdmFpbGFibGUgYW5kIGFyZSB1c2VkIGluIGRpZmZlcmVudCBwbGFjZXMgaW4gdGhlIHdvcmxkIHRvIG1hbmFnZSBjcm9wcyBhbmQgc29pbHMuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDcuMzkwMTYzNDIxNjMwODYsImxlZnQiOjkuNTIzODAwODQ5OTE0NTUsIndpZHRoIjozOS4xNjYyOTAyODMyMDMxMjUsImhlaWdodCI6MS4yMTAyNzc5MTUwMDA5MTU1fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDguNjEyNjEzNjc3OTc4NTE2LCJsZWZ0Ijo5LjUyMzgwMDg0OTkxNDU1LCJ3aWR0aCI6MzkuMTYyODcyMzE0NDUzMTI1LCJoZWlnaHQiOjEuMjEwMjc3OTE1MDAwOTE1NX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQ5LjgzNTA1MjQ5MDIzNDM3NSwibGVmdCI6OS41MjM4MDA4NDk5MTQ1NSwid2lkdGgiOjM5LjE2NjE0NTMyNDcwNzAzLCJoZWlnaHQiOjEuMjEwMjc3OTE1MDAwOTE1NX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjUxLjA1NzQ5NTExNzE4NzUsImxlZnQiOjkuNTIzODAwODQ5OTE0NTUsIndpZHRoIjoxMy40MTU0MjYyNTQyNzI0NjEsImhlaWdodCI6MS4yMTAyNzc5MTUwMDA5MTU1fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTIuOTUzMzMwOTkzNjUyMzQ0LCJsZWZ0Ijo5LjUyMzgwMDg0OTkxNDU1LCJ3aWR0aCI6MzkuMTY4MjgxNTU1MTc1NzgsImhlaWdodCI6MTMuNDM0NzE1MjcwOTk2MDk0fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NjcuMTk1OTQ1NzM5NzQ2MSwibGVmdCI6OS41MjM4MDA4NDk5MTQ1NSwid2lkdGgiOjE3LjE1Njk2NTI1NTczNzMwNSwiaGVpZ2h0IjoxLjcxMDAxOTU4ODQ3MDQ1OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjY5LjE1NTM5NTUwNzgxMjUsImxlZnQiOjkuNTIzODAwODQ5OTE0NTUsIndpZHRoIjozOS4xNjczMDExNzc5Nzg1MTYsImhlaWdodCI6Ni4xMDAwNDk5NzI1MzQxOH19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjc1Ljk0MTAwMTg5MjA4OTg0LCJsZWZ0Ijo5LjUyMzgwMDg0OTkxNDU1LCJ3aWR0aCI6MzkuMTYyMTQ3NTIxOTcyNjU2LCJoZWlnaHQiOjEuMjEwMjc3OTE1MDAwOTE1NX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjc3LjE2MzQ1OTc3NzgzMjAzLCJsZWZ0Ijo5LjUyMzgwMDg0OTkxNDU1LCJ3aWR0aCI6MzkuMTYyNzE1OTExODY1MjM0LCJoZWlnaHQiOjEuMjEwMjc3OTE1MDAwOTE1NX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjUifX0seyJvcmlnaW4iOjQsIndvcmsiOnsidHlwZSI6MzAsInN1YnR5cGUiOnt9LCJjb250cmlidXRvcnMiOlt7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJRaWFuZyIsImZhbWlseSI6IkNoYWkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiWWFudGFpIiwiZmFtaWx5IjoiR2FuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkNhaSIsImZhbWlseSI6IlpoYW8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSHVpLWxpYW4iLCJmYW1pbHkiOiJYdSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJSZWFnYW4iLCJmYW1pbHkiOiJXYXNrb20ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiWWluaW5nIiwiZmFtaWx5IjoiTml1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkthZGFtYm90IEguIE0uIiwiZmFtaWx5IjoiU2lkZGlxdWUifX1dLCJpZHMiOnsiaXNzbiI6WyIxNzczLTAxNTUiLCIxNzc0LTA3NDYiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMDcvczEzNTkzLTAxNS0wMzM4LTYiLCJtYWciOiIyMjIzOTYxMjgzIiwib3BlbmFsZXgiOiJXMjIyMzk2MTI4MyJ9LCJ0aXRsZSI6IlJlZ3VsYXRlZCBkZWZpY2l0IGlycmlnYXRpb24gZm9yIGNyb3AgcHJvZHVjdGlvbiB1bmRlciBkcm91Z2h0IHN0cmVzcy4gQSByZXZpZXciLCJhYnN0cmFjdCI6IkFncmljdWx0dXJlIGNvbnN1bWVzIG1vcmUgdGhhbiB0d28gdGhpcmRzIG9mIHRoZSB0b3RhbCBmcmVzaHdhdGVyIG9mIHRoZSBwbGFuZXQuIFRoaXMgaXNzdWUgY2F1c2VzIHN1YnN0YW50aWFsIGNvbmZsaWN0IGluIGZyZXNod2F0ZXIgYWxsb2NhdGlvbiBiZXR3ZWVuIGFncmljdWx0dXJlIGFuZCBvdGhlciBlY29ub21pYyBzZWN0b3JzLiBSZWd1bGF0ZWQgZGVmaWNpdCBpcnJpZ2F0aW9uIChSREkpIGlzIGtleSB0ZWNobm9sb2d5IGJlY2F1c2UgaXQgaGVscHMgdG8gaW1wcm92ZSB3YXRlciB1c2UgZWZmaWNpZW5jeS4gTm9uZXRoZWxlc3MsIHRoZXJlIGlzIGEgbGFjayBvZiB1bmRlcnN0YW5kaW5nIG9mIHRoZSBtZWNoYW5pc21zIHdpdGggd2hpY2ggcGxhbnRzIHJlc3BvbmQgdG8gUkRJLiBJbiBwYXJ0aWN1bGFyLCBsaXR0bGUgaXMga25vd24gYWJvdXQgaG93IFJESSBtaWdodCBpbmNyZWFzZSBjcm9wIHByb2R1Y3Rpb24gd2hpbGUgcmVkdWNpbmcgdGhlIGFtb3VudCBvZiBpcnJpZ2F0aW9uIHdhdGVyIGluIHJlYWwtd29ybGQgYWdyaWN1bHR1cmUuIEluIHRoaXMgcmV2aWV3LCB3ZSBmb3VuZCB0aGF0IFJESSBpcyBsYXJnZWx5IGltcGxlbWVudGVkIHRocm91Z2ggdGhyZWUgYXBwcm9hY2hlczogKDEpIGdyb3d0aCBzdGFnZS1iYXNlZCBkZWZpY2l0IGlycmlnYXRpb24sICgyKSBwYXJ0aWFsIHJvb3Qtem9uZSBpcnJpZ2F0aW9uLCBhbmQgKDMpIHN1YnN1cmZhY2UgZHJpcHBlciBpcnJpZ2F0aW9uLiBBbW9uZyB0aGVzZSwgcGFydGlhbCByb290LXpvbmUgaXJyaWdhdGlvbiBpcyB0aGUgbW9zdCBwb3B1bGFyIGFuZCBlZmZlY3RpdmUgYmVjYXVzZSBtYW55IGZpZWxkIGNyb3BzIGFuZCBzb21lIHdvb2R5IGNyb3BzIGNhbiBzYXZlIGlycmlnYXRpb24gd2F0ZXIgdXAgdG8gMjAgdG8gMzAgJSB3aXRob3V0IG9yIHdpdGggYSBtaW5pbWFsIGltcGFjdCBvbiBjcm9wIHlpZWxkLiBJbXByb3ZlZCB3YXRlciB1c2UgZWZmaWNpZW5jeSB3aXRoIFJESSBpcyBtYWlubHkgZHVlIHRvIHRoZSBmb2xsb3dpbmc6ICgxKSBlbmhhbmNlZCBndWFyZCBjZWxsIHNpZ25hbCB0cmFuc2R1Y3Rpb24gbmV0d29yayB0aGF0IGRlY3JlYXNlcyB0cmFuc3BpcmF0aW9uIHdhdGVyIGxvc3MsICgyKSBvcHRpbWl6ZWQgc3RvbWF0YWwgY29udHJvbCB0aGF0IGltcHJvdmVzIHRoZSBwaG90b3N5bnRoZXNpcyB0byB0cmFuc3BpcmF0aW9uIHJhdGlvLCBhbmQgKDMpIGRlY3JlYXNlZCBldmFwb3JhdGl2ZSBzdXJmYWNlIGFyZWFzIHdpdGggcGFydGlhbCByb290LXpvbmUgaXJyaWdhdGlvbiB0aGF0IHJlZHVjZXMgc29pbCBldmFwb3JhdGlvbi4gVGhlIG1lY2hhbmlzbXMgaW52b2x2ZWQgaW4gdGhlIHBsYW50IHJlc3BvbnNlIHRvIFJESS1pbmR1Y2VkIHdhdGVyIHN0cmVzcyBpbmNsdWRlIHRoZSBtb3JwaG9sb2dpY2FsIHRyYWl0cywgZS5nLiwgaW5jcmVhc2VkIHJvb3QgdG8gc2hvb3QgcmF0aW8gYW5kIGltcHJvdmVkIG51dHJpZW50IHVwdGFrZSBhbmQgcmVjb3Zlcnk7IHBoeXNpb2xvZ2ljYWwgdHJhaXRzLCBlLmcuLCBzdG9tYXRhbCBjbG9zdXJlLCBkZWNyZWFzZWQgbGVhZiByZXNwaXJhdGlvbiwgYW5kIG1haW50YWluZWQgcGhvdG9zeW50aGVzaXM7IGFuZCBiaW9jaGVtaWNhbCB0cmFpdHMsIGUuZy4sIGluY3JlYXNlZCBzaWduYWxpbmcgbW9sZWN1bGVzIGFuZCBlbmhhbmNlZCBhbnRpb3hpZGF0aW9uIGVuenltYXRpYyBhY3Rpdml0eS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAwNy9zMTM1OTMtMDE1LTAzMzgtNiIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9saW5rLnNwcmluZ2VyLmNvbS9jb250ZW50L3BkZi8xMC4xMDA3L3MxMzU5My0wMTUtMDMzOC02LnBkZiIsImlzc3VlZERhdGUiOiIyMDE1LTEyLTE4IiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiQWdyb25vbXkgZm9yIFN1c3RhaW5hYmxlIERldmVsb3BtZW50IiwiaXNzdWUiOjEsInZvbHVtZSI6MzYsImltcGFjdEZhY3RvciI6IjkuNzYifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiU3ByaW5nZXIgU2NpZW5jZStCdXNpbmVzcyBNZWRpYSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MTQ4LCJjaXRlZEJ5Q291bnQiOjU1M319LCJxdW90ZSI6IkFkZGl0aW9uYWxseSwgdGhlIHJlYWxsb2NhdGlvbiBvZiB3YXRlciByZXNvdXJjZXMgYW5kIG1vZGlmaWNhdGlvbiBvZiBpcnJpZ2F0aW9uIHN5c3RlbXMgbWF5IGhhdmUgYSByb2xlIHRvIHBsYXkgZm9yIHRoZSBpbXByb3ZlbWVudCBvZiB3YXRlciB1c2UgaW4gYWdyaWN1bHR1cmUgKEhvbWF5b3VuZmFyIGV0IGFsLiAyMDE0KS4gNy4yIENoYWxsZW5nZXMgVGhlIGNlbnRyYWwgbm92ZWx0eSBvZiBSREkgaXMgdGhhdCBpdCBhbGxvd3MgcGxhbnRzIHRvIGdyb3cgdW5kZXIgbWlsZCB3YXRlciBzdHJlc3MgdG8gaW5kdWNlIHJvb3Qtc291cmNlZCBjaGVtaWNhbCBzaWctbmFscyBmcm9tIGRyeWluZyByb290cyB0byByZWFjaCB0aGUgc2hvb3Qgd2hlcmUgcGh5c2lvbG9naWNhbCBhbmQgYmlvY2hlbWljYWwgcHJvY2Vzc2VzIGNhbiBiZSByZWd1bGF0ZWQuIEhvd2V2ZXIsIHRoZSBwaHlzaW9sb2dpY2FsIGFuZCBiaW9jaGVtaWNhbCByZXNwb25zZXMgYXJlIGRpZmZpY3VsdCB0byBxdWFuLXRpZnksIGdpdmVuIHRoYXQgcGFydGlhbCByb290LXpvbmUgaXJyaWdhdGlvbiBpcyB1c3VhbGx5IGFwcGxpZWQgdG8gcGxhbnRzIGluIGFjY29yZGFuY2Ugd2l0aCBib3RoIHRlbXBvcmFsIGFuZCBzcGF0aWFsIGRyb3VnaHRzLiBJbiBwcmFjdGljZSwgYWRvcHRpb24gb2Ygd2F0ZXItc2F2aW5nIHRlY2huaXF1ZSByZXF1aXJlcyBzY2ktZW50aWZpYyB1bmRlcnN0YW5kaW5nIG9mIHRoZSBtZWNoYW5pc21zIGludm9sdmVkIGluIHRoZSBwaHlzaW9sb2dpY2FsIGFuZCBiaW9jaGVtaWNhbCBwcm9jZXNzZXMuIEFsc28sIHRoZSBvdXRjb21lIG9mIHRoZSB3YXRlciBzdHJlc3MtaW5kdWNlZCByZXNwb25zZXMgaXMgaW5mbHVlbmNlZCBieSBtYW55IGZhY3RvcnMsIHN1Y2ggYXMgdGhlIHN0YWdlcyBvZiBjcm9wIGdyb3d0aCB3aGVuIHRoZSBkZWZpY2l0IGlzIGFwcGxpZWQsIHRoZSBpbnRlbnNpdHkgYW5kIHNldmVyaXR5IG9mIGRlZmljaXQgaW1wb3NlZCwgdGhlIG1ldGhvZCB3aXRoIHdoaWNoIGRlZmljaXQgaXJyaWdhdGlvbiBpcyBhcHBsaWVkLCBjcm9wIHNwZWNpZXMgYW5kIGN1bHRpdmFycywgYW5kIHRoZWlyIHJlc3BlY3RpdmUgbm9uLWNyaXRpY2FsIGdyb3d0aCBzdGFnZXMgd2hlbiB0aGUgc3RhZ2UtYmFzZWQgZGVmaWNpdCBpcnJpZ2F0aW9uIG1heSBiZSBhcHBsaWVkLiBUaGVzZSBmYWN0b3JzIG5lZWQgdG8gYmUgaW52ZXN0aWdhdGVkIGF0IGEgc3lzdGVtIGxldmVsIHVuZGVyIHNwZWNpZmljIGdyb3dpbmcgY29uZGl0aW9ucy4gUkRJIGFuZCwgcGFydGljdWxhcmx5LCBhbHRlcm5hdGUgcGFydGlhbCByb290LXpvbmUgaXJyaWdhdGlvbiBhcmUgc3RpbGwgcmVsYXRpdmVseSBuZXcgdG8gbWFuYWdlIGZhcm0gbWFuYWdlcnMuIFRoaXMgdGVjaC1uaXF1ZSBpcyBub3QgZWFzeSB0byBpbXBsZW1lbnQgaW4gY3JvcCBzcGVjaWVzIHdpdGggZGVuc2UgcGxhbnQgcG9wdWxhdGlvbiBzdWNoIGFzIGNlcmVhbCBhbmQgb2lsc2VlZCBjcm9wcy4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEzLCJiYm94Ijp7InRvcCI6NjUuNTQxNDgxMDE4MDY2NCwibGVmdCI6NTEuNDMxODQ2NjE4NjUyMzQ0LCJ3aWR0aCI6MzkuOTk0MjgxNzY4Nzk4ODMsImhlaWdodCI6MS4zNjQyNjI3MDAwODA4NzE2fX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjY3LjExODY0NDcxNDM1NTQ3LCJsZWZ0Ijo1MS40MzE4NDY2MTg2NTIzNDQsIndpZHRoIjozOS45OTU5NDExNjIxMDkzNzUsImhlaWdodCI6MS4zNjQyNjI3MDAwODA4NzE2fX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjY4LjY4ODI0MDA1MTI2OTUzLCJsZWZ0Ijo1MS40MzE4NDY2MTg2NTIzNDQsIndpZHRoIjozOS45OTI1ODA0MTM4MTgzNiwiaGVpZ2h0IjoxLjM2NDI2MjcwMDA4MDg3MTZ9fSx7InBhZ2VOdW1iZXIiOjEzLCJiYm94Ijp7InRvcCI6NzAuMjY1Mzk2MTE4MTY0MDYsImxlZnQiOjUxLjQzMTg0NjYxODY1MjM0NCwid2lkdGgiOjE3Ljg5MDg5MjAyODgwODU5NCwiaGVpZ2h0IjoxLjM2NDI2MjcwMDA4MDg3MTZ9fSx7InBhZ2VOdW1iZXIiOjEzLCJiYm94Ijp7InRvcCI6NzMuNDA3MTA0NDkyMTg3NSwibGVmdCI6NTEuNDMxODQ2NjE4NjUyMzQ0LCJ3aWR0aCI6MTAuMTAzNTkxOTE4OTQ1MzEyLCJoZWlnaHQiOjEuMzc2ODU3NjM4MzU5MDY5OH19LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjo3Ni41NjY0NTk2NTU3NjE3MiwibGVmdCI6NTEuNDMxODQ2NjE4NjUyMzQ0LCJ3aWR0aCI6NDAuMDA3NjcxMzU2MjAxMTcsImhlaWdodCI6MTMuOTU4ODIwMzQzMDE3NTc4fX0seyJwYWdlTnVtYmVyIjoxNCwiYmJveCI6eyJ0b3AiOjcuMjg0ODU4NzAzNjEzMjgxLCJsZWZ0Ijo4LjU3MTQxODc2MjIwNzAzMSwid2lkdGgiOjQwLjAwNzY0MDgzODYyMzA1LCJoZWlnaHQiOjEuMzY0MjYzMTc2OTE4MDI5OH19LHsicGFnZU51bWJlciI6MTQsImJib3giOnsidG9wIjo4Ljg1NDQ1MzA4Njg1MzAyNywibGVmdCI6OC41NzE0MTg3NjIyMDcwMzEsIndpZHRoIjo0MC4wMTU5OTUwMjU2MzQ3NjYsImhlaWdodCI6MS4zNjQyNjM2NTM3NTUxODh9fSx7InBhZ2VOdW1iZXIiOjE0LCJiYm94Ijp7InRvcCI6MTAuNDMxNjExMDYxMDk2MTkxLCJsZWZ0Ijo4LjU3MTQxODc2MjIwNzAzMSwid2lkdGgiOjQwLjAwNzY0MDgzODYyMzA1LCJoZWlnaHQiOjEuMzY0MjYzNjUzNzU1MTg4fX0seyJwYWdlTnVtYmVyIjoxNCwiYmJveCI6eyJ0b3AiOjEyLjAwODc2MTQwNTk0NDgyNCwibGVmdCI6OC41NzE0MTg3NjIyMDcwMzEsIndpZHRoIjo0MC4wMDQzMDI5Nzg1MTU2MjUsImhlaWdodCI6MS4zNjQyNjM2NTM3NTUxODh9fSx7InBhZ2VOdW1iZXIiOjE0LCJiYm94Ijp7InRvcCI6MTMuNTc4MzU1Nzg5MTg0NTcsImxlZnQiOjguNTcxNDE4NzYyMjA3MDMxLCJ3aWR0aCI6MzkuOTk3NTk2NzQwNzIyNjU2LCJoZWlnaHQiOjEuMzY0MjYzNjUzNzU1MTg4fX0seyJwYWdlTnVtYmVyIjoxNCwiYmJveCI6eyJ0b3AiOjE1LjE1NTUxMzc2MzQyNzczNCwibGVmdCI6OC41NzE0MTg3NjIyMDcwMzEsIndpZHRoIjo0MC4wMTA5OTM5NTc1MTk1MywiaGVpZ2h0IjoxLjM2NDI2MzY1Mzc1NTE4OH19LHsicGFnZU51bWJlciI6MTQsImJib3giOnsidG9wIjoxNi43MzI2NjQxMDgyNzYzNjcsImxlZnQiOjguNTcxNDE4NzYyMjA3MDMxLCJ3aWR0aCI6NDAuMDEwOTkwMTQyODIyMjY2LCJoZWlnaHQiOjEuMzY0MjYyNzAwMDgwODcxNn19LHsicGFnZU51bWJlciI6MTQsImJib3giOnsidG9wIjoxOC4zMDIyNTk0NDUxOTA0MywibGVmdCI6OC41NzE0MTg3NjIyMDcwMzEsIndpZHRoIjo0MC4wMDc2MzcwMjM5MjU3OCwiaGVpZ2h0IjoxLjM2NDI2MjcwMDA4MDg3MTZ9fSx7InBhZ2VOdW1iZXIiOjE0LCJiYm94Ijp7InRvcCI6MTkuODc5NDE3NDE5NDMzNTk0LCJsZWZ0Ijo4LjU3MTQxODc2MjIwNzAzMSwid2lkdGgiOjEzLjA4OTMwNDkyNDAxMTIzLCJoZWlnaHQiOjEuMzY0MjYyNzAwMDgwODcxNn19LHsicGFnZU51bWJlciI6MTQsImJib3giOnsidG9wIjoyMS40NTY1Njc3NjQyODIyMjcsImxlZnQiOjEwLjQ3NTk4NzQzNDM4NzIwNywid2lkdGgiOjM4LjExMTQyNzMwNzEyODkwNiwiaGVpZ2h0IjoxLjM2NDI2MjcwMDA4MDg3MTZ9fSx7InBhZ2VOdW1iZXIiOjE0LCJiYm94Ijp7InRvcCI6MjMuMDI2MTYzMTAxMTk2MjksImxlZnQiOjguNTcxNDE4NzYyMjA3MDMxLCJ3aWR0aCI6MzkuOTkyNTg4MDQzMjEyODksImhlaWdodCI6MS4zNjQyNjI3MDAwODA4NzE2fX0seyJwYWdlTnVtYmVyIjoxNCwiYmJveCI6eyJ0b3AiOjI0LjYwMzMyMTA3NTQzOTQ1MywibGVmdCI6OC41NzE0MTg3NjIyMDcwMzEsIndpZHRoIjo0MC4wMDI2MTMwNjc2MjY5NSwiaGVpZ2h0IjoxLjM2NDI2MjcwMDA4MDg3MTZ9fSx7InBhZ2VOdW1iZXIiOjE0LCJiYm94Ijp7InRvcCI6MjYuMTgwNDY5NTEyOTM5NDUzLCJsZWZ0Ijo4LjU3MTQxODc2MjIwNzAzMSwid2lkdGgiOjM5Ljk5NTkyMjA4ODYyMzA1LCJoZWlnaHQiOjEuMzY0MjYyNzAwMDgwODcxNn19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjE0In19XQ%3D%3D.

## Conclusion

Advanced irrigation technologies offer transformative potential for water efficiency and sustainability, yet overcoming technological, economic, and climatic barriers through policy support, inclusive innovation, and targeted research is imperative for global food security. Further efforts should focus on integrating advanced irrigation with drought-resistant crop varieties and sophisticated hydrological modeling to maximize water savings and agricultural output under changing climatic conditions.

References

- Ali, A., Hussain, T., & Zahid, A. (2025). Smart Irrigation Technologies and Prospects for Enhancing Water Use Efficiency for Sustainable Agriculture. AgriEngineering, 7(4), 106. https://doi.org/10.3390/agriengineering7040106

- Antu, U. B., Islam, Md. S., Ahmed, S., Arifuzzaman, M., Saha, S. C. S. and N. N., Mitu, P. R., Sarkar, A., Mahiddin, N. A., Ismail, Z., Ibrahim, K. A., & Idris, A. M. (2024). Emerging technologies for efficient water use in agriculture: A review of current trends and future directions [Review of Emerging technologies for efficient water use in agriculture: A review of current trends and future directions]. Journal of Water Process Engineering, 68, 106317. Elsevier BV. https://doi.org/10.1016/j.jwpe.2024.106317

- Borin, M. (2024). A wise irrigation to contribute to integrated water resource management. Italian Journal of Agrometeorology, 2, 5. https://doi.org/10.36253/ijam-2323

- Chai, Q., Gan, Y., Zhao, C., Xu, H., Waskom, R., Niu, Y., & Siddique, K. H. M. (2015). Regulated deficit irrigation for crop production under drought stress. A review [Review of Regulated deficit irrigation for crop production under drought stress. A review]. Agronomy for Sustainable Development, 36(1). Springer Science+Business Media. https://doi.org/10.1007/s13593-015-0338-6

- Chami, D. E., &Moujabber, M. E. (2016). Drought, climate change and sustainability of water in agriculture: A roadmap towards the NWRS2. South African Journal of Science, 112, 4. https://doi.org/10.17159/sajs.2016/20150457

- Daraz, U., Bojnec, Š., & Khan, Y. (2025). Energy-Efficient Smart Irrigation Technologies: A Pathway to Water and Energy Sustainability in Agriculture. Agriculture, 15(5), 554. https://doi.org/10.3390/agriculture15050554

- Evans, R. G., & Sadler, E. J. (2008). Methods and technologies to improve efficiency of water use. Water Resources Research, 44(7). https://doi.org/10.1029/2007wr006200

- Fader, M., Giupponi, C., Burak, S., Dakhlaoui, H., Koutroulis, A., Lange, M. A., Llasat, M. C., Pulido‐Velazquez, D., & Sanz-Cobeña, A. (2020). Chapter 3 Resources - Subchapter 3.1 Water. HAL (Le Centre Pour La Communication ScientifiqueDirecte). https://hal-amu.archives-ouvertes.fr/hal-03210040

- Gomes, M., Ralph, T. J., Humphries, M., Graves, B. P., Kobayashi, T., & Gore, D. B. (2024). Waterborne contaminants in high intensity agriculture and plant production: A review of on-site and downstream impacts [Review of Waterborne contaminants in high intensity agriculture and plant production: A review of on-site and downstream impacts]. The Science of The Total Environment, 958, 178084. Elsevier BV. https://doi.org/10.1016/j.scitotenv.2024.178084

- Hashemi, S., Darzi‐Naftchali, A., Karandish, F., Ritzema, H. P., &Solaimani, K. (2024). Enhancing agricultural sustainability with water and crop management strategies in modern irrigation and drainage networks. Agricultural Water Management, 305, 109110. https://doi.org/10.1016/j.agwat.2024.109110

- Hla, A. K., & Scherer, T. F. (2003). Introduction to Micro-irrigation. NDSU Repository (North Dakota State University). http://hdl.handle.net/10365/5108

- Jumintono, J., Nurnawaty, N., Brata, N. T., Rahmiati, D., Susilo, H., &Hussen, A. A. (2024). Comparative analysis of precision, deficit, and drip irrigation for water efficiency in Iraq. Journal of Water and Land Development, 130. https://doi.org/10.24425/jwld.2024.151798

- Kushwaha, M., Singh, S., Singh, V. K., & Dwivedi, S. (2024). Precision Farming: A Review of Methods, Technologies, and Future Prospects [Review of Precision Farming: A Review of Methods, Technologies, and Future Prospects]. International Journal of Environment Agriculture and Biotechnology, 9(2), 242. Infogain Publication. https://doi.org/10.22161/ijeab.92.27

- Mahmood, H., Hassan, M. S., Meraj, G., & Furqan, M. (2024). Agriculture’s Role in Environmental Sustainability: A Comprehensive Review of Challenges and Solutions [Review of Agriculture’s Role in Environmental Sustainability: A Comprehensive Review of Challenges and Solutions]. Challenges in Sustainability, 12(3), 178. Librelloph. https://doi.org/10.56578/cis120302

- Mansoor, S., Iqbal, S., Popescu, S. M., Kim, S. L., Mansoor, S., &Baek, J. (2025). Integration of smart sensors and IOT in precision agriculture: trends, challenges and future prospectives [Review of Integration of smart sensors and IOT in precision agriculture: trends, challenges and future prospectives]. Frontiers in Plant Science, 16, 1587869. Frontiers Media. https://doi.org/10.3389/fpls.2025.1587869

- Pérez‐Blanco, C. D., Essenfelder, A. H., & Perry, C. (2020). Irrigation Technology and Water Conservation: A Review of the Theory and Evidence [Review of Irrigation Technology and Water Conservation: A Review of the Theory and Evidence]. Review of Environmental Economics and Policy, 14(2), 216. University of Chicago Press. https://doi.org/10.1093/reep/reaa004

- Plaščak, I., Jurišić, M., Radočaj, D., Vujić, M., & Zimmer, D. (2021). An Overview of Precision Irrigation Systems Used in Agriculture. TehničkiGlasnik, 15(4), 546. https://doi.org/10.31803/tg-20210416103500

- Qin, J., Duan, W., Zou, S., Chen, Y., Huang, W., & Rosa, L. (2024). Global energy use and carbon emissions from irrigated agriculture. Nature Communications, 15(1), 3084. https://doi.org/10.1038/s41467-024-47383-5

- Saha, G., Shahrin, F., Khan, F. H., Meshkat, M. A., & Azad, A. A. M. (2025). Smart IoT-driven precision agriculture: Land mapping, crop prediction, and irrigation system. PLoS ONE, 20(3). https://doi.org/10.1371/journal.pone.0319268

- Sharma, R. K., Nwosu, N. J., Singh, L., Kramer, A., Singh, H., Bheemanahalli, R., & Dhillon, J. (2025). Agricultural sustainability under unpredicted atmospheric changes—strategies to enhance crop resilience and system efficiency: a narrative review [Review of Agricultural sustainability under unpredicted atmospheric changes—strategies to enhance crop resilience and system efficiency: a narrative review]. Discover Agriculture, 3(1). Springer Science+Business Media. https://doi.org/10.1007/s44279-025-00287-4

- Shetty, G.B., Ananthakumar, M.A., Bhagyalakshmi, T. and Yogananda, S.B., 2023. Influence of levels of water soluble fertilizer on yield, quality and economics of sugarcane through fertigation. Int. J. Plant Soil Sci., 35(21):277–285.

- Timilsina, R. R., Khosla, S. K., Rahut, D. B., Jena, P. R., &Sonobe, T. (2025). A comprehensive review of the soil health status for enhancing agricultural sustainability [Review of A comprehensive review of the soil health status for enhancing agricultural sustainability]. Frontiers in Environmental Science, 13. Frontiers Media. https://doi.org/10.3389/fenvs.2025.1548095

- Walter, A., Finger, R., Huber, R., & Buchmann, N. (2017). Smart farming is key to developing sustainable agriculture. Proc. Nat. Acad. Sci. USA, 114, 6148. https://doi.org/10.1073/pnas.1707462114

- Wu, S., Wei, Y., Head, B., Zhao, Y., & Hanna, S. (2019). The development of ancient Chinese agricultural and water technology from 8000 BC to 1911 AD. Palgrave Communications, 5(1). https://doi.org/10.1057/s41599-019-0282-1

- Xie, C. (2025). The role of modern agricultural technologies in improving agricultural productivity and land use efficiency [Review of The role of modern agricultural technologies in improving agricultural productivity and land use efficiency]. Frontiers in Plant Science, 16. Frontiers Media. https://doi.org/10.3389/fpls.2025.1675657