A Multi-Scenario Analysis on Techno- Economic Assessment of Groundnut Shell Gasification for Decentralised Power Generation in Senegal

Abstract

Senegal’s rural electrification rate remains below 55%, with off-grid communities heavily dependent on diesel generation at costs of 190–220 FCFA/kWh. Groundnut shell, a lignocellulosic agricultural residue available at an estimated 700,000–900,000 tonnes annually across Senegal’s groundnut basin, represents a viable feedstock for decentralised gasification-based power generation. The aim of this paper to presents a multi-scenario analysis of first country-specific techno-economic assessment of a 500 kWₑ downdraft gasification system fuelled by groundnut shell for off-grid rural electrification in Senegal. Technical performance parameters, feedstock characterisation data and plant cost estimates were derived from literature proxies for analogous systems in comparable Sub-Saharan African contexts. Three development scenarios were evaluated: conservative (S1), base case (S2), and optimistic (S3), differing in capacity factor, feedstock cost and electricity tariff. The results indicate a levelised cost of electricity (LCOE) ranging from 98 to 142 FCFA/kWh, a net present value (NPV) of 48 to 187 million FCFA and an internal rate of return (IRR) of 14.2% to 22.8% across scenarios, with a discounted payback period of 4.2 to 7.8 years. The sensitivity analysis confirms that feedstock cost is the dominant financial driver, with a ±30% variation inducing a ±18% change in IRR. At the base case LCOE of 112 FCFA/kWh, the technology demonstrates a decisive cost advantage over diesel generation, supporting its deployment under PERACCU-aligned rural electrification programmes and potential carbon credit monetisation under voluntary carbon market frameworks.

The theoretical and practical implications are…

The contribution of study is…

Keywords: Groundnut shell; Biomass gasification; Techno-economic assessment; Rural electrification; Power generations

# 1. Introduction

Access to affordable and reliable electricity remains one of the most pressing development challenges across sub-Saharan Africa. In Senegal, despite significant recent progress driven by the Plan Sénégal Émergent (PSE) and the Programme d’Électrification Rurale et d’Accès aux Combustibles de Cuisson Universels (PERACCU), rural electrification rates remain below 55%, with deep rural communities still relying on diesel generators operating at tariffs of 190–220 FCFA/kWh [1,2]. This energy access gap carries direct economic, social and environmental consequences, particularly for agricultural processing industries, small and medium enterprises, and health facilities in the groundnut basin regions of Kaolack, Fatick and Kaffrine.

Biomass gasification, the thermochemical conversion of solid biomass into a combustible producer gas (syngas) through partial oxidation at temperatures of 700–1200°C, offers a technically mature and financially scalable pathway for decentralised power generation [3,4]. Downdraft fixed-bed gasifiers, operating at capacities of 10–1000 kWₑ, are particularly well-suited to off-grid rural applications due to their relatively simple operation, low tar content in the producer gas and compatibility with conventional internal combustion generator sets [5]. Groundnut shell (Arachis hypogaea), characterised by a bulk density of 80–130 kg/m³, ash content of 2–6% and higher heating value of 15.5–18.5 MJ/kg [6-7], exhibits physico-chemical properties consistent with successful gasification in downdraft configurations. The feedstock is abundantly available in Senegal at negligible or negative cost in major producing regions, where it currently represents a waste disposal burden for oil milling facilities [8].

The techno-economic feasibility of biomass gasification for rural electrification has been documented across a range of Sub-Saharan African contexts, including rice husk in West Africa [9], sugarcane bagasse in East Africa [10], and wood chips in Southern Africa [11]. However, groundnut shell gasification has received comparatively limited dedicated techno-economic analysis, with existing studies predominantly focused on South Asian contexts [12-13] where biomass costs, labour rates, grid electricity prices and financing conditions differ substantially from those prevailing in Senegal. To the best of the authors’ knowledge, no peer-reviewed techno-economic assessment of groundnut shell gasification has been published for Senegalese or broader West African conditions, representing a significant gap in the evidence base available to energy project developers and policymakers in the region.

This study addresses this gap by presenting a multi-scenario techno-economic assessment of a 500 kWₑ downdraft gasification plant fuelled by groundnut shell for off-grid rural electrification in Senegal. The specific objectives are: (i) to define the technical configuration and performance parameters of the reference gasification system; (ii) to estimate capital expenditure (CAPEX), operational expenditure (OPEX) and the levelised cost of electricity (LCOE) under Senegalese market conditions; (iii) to evaluate financial viability indicators (NPV, IRR, payback period) across three development scenarios; and (iv) to identify the primary financial sensitivity drivers through a quantitative sensitivity analysis. The novelty of this work lies in its first-of-kind application of country-specific techno-economic modelling to groundnut shell gasification in Senegal, integrating local feedstock cost data, national financing parameters and regulatory context. The structure of the paper is as follows: Section 2 describes the system configuration and technical performance parameters. Section 3 presents the economic modelling framework and scenario definitions. Results are reported in Section 4, followed by discussion in Section 5 and conclusions in Section 6.

2.Materials and Method

# System Configuration and Technical Parameters

## 2.1 Reference System: 500 kWₑ Downdraft Gasification Plant

The table 1 below show the reference system consists of a 500 kWₑ net electrical output downdraft fixed-bed gasifier integrated with a producer gas cleaning train (cyclone, wet scrubber, fabric filter) and a 625 kVA gas-fuelled generator set. A 10% auxiliary power consumption is assumed, yielding a net electrical output of 500 kWₑ. The system is designed for continuous operation at a nominal capacity factor of 75% (base case), corresponding to 6,570 operating hours per year. The feedstock consumption rate is estimated at 1.8–2.2 kg/kWhₑ based on a cold gas efficiency of 70–75% and a gasification agent-to-biomass ratio of 0.28–0.32 (proxies from analogous downdraft systems in comparable climatic conditions) [14-15].

Table 1. Key Technical Parameters of the Reference 500 kWₑ Groundnut Shell Gasification System

| Parameter | Unit | Value / Range | Source |
| --- | --- | --- | --- |
| Installed capacity (gross) | kWₑ | 625 | [14,15] |
| Net electrical output | kWₑ | 500 | design |
| Auxiliary consumption | % | 10 | assumption |
| Cold gas efficiency | % | 70–75 | [14,16] |
| Gasification temperature | °C | 900–1050 | [14,16] |
| Equivalence ratio (ER) | — | 0.28–0.32 | [16,17] |
| Producer gas LHV | MJ/Nm³ | 4.2–5.8 | [14,15,16] |
| Feedstock consumption rate | kg/kWhₑ | 1.8–2.2 | calc. |
| Annual feedstock requirement (CF=75%) | tonne/year | ~1,180–1,450 | calc. |
| Biochar by-product yield | % wt feedstock | 5–8 | [17,18] |
| Plant design life | years | 20 | standard |
| Capacity factor — S1 / S2 / S3 | % | 60 / 75 / 85 | scenarios |

Note : calc.: calculated from technical parameters above; CF: capacity factor; S1–S3: development scenarios.

Source: Research Findings (2026)

## 2.2 Feedstock Supply and Logistics

Groundnut shell supply was modelled for a plant location in the Kaolack region, within 80 km of the major groundnut processing clusters. Three feedstock procurement scenarios were defined: (i) S1 — commercial purchase from aggregators at 18,000 FCFA/tonne; (ii) S2 (base case) — mixed procurement combining direct sourcing from oil mills (50%) at 10,000 FCFA/tonne with commercial purchase at 18,000 FCFA/tonne, yielding an effective cost of 14,000 FCFA/tonne; and (iii) S3 — direct partnership with three major groundnut processing cooperatives supplying at 8,000 FCFA/tonne under a long-term supply agreement. These cost ranges are consistent with agricultural residue pricing documented for the Groundnut Basin region [8,19] and comparable to biomass feedstock costs reported for rural gasification projects in West Africa [9].

# 3. Economic Modelling Framework

## 3.1 Capital Expenditure (CAPEX) Estimation

Capital expenditure was estimated using a combination of cost functions from the biomass gasification literature [20,21] adjusted for Senegalese market conditions (import duties, installation costs, civil works) using regional cost adjustment factors (RCAF) of 1.15–1.25 relative to European reference costs [22]. The CAPEX structure is detailed in Table 2 below.

Table 2. CAPEX Structure for the 500 kWₑ Groundnut Shell Gasification Plant (Base Case, million FCFA)

| Cost Component | S1 — Conservative | S2 — Base Case | S3 — Optimistic | Notes |
| --- | --- | --- | --- | --- |
| Gasifier unit (supply + erection) | 142.5 | 128.0 | 118.0 | imported, CIF Dakar |
| Gas cleaning train | 52.0 | 46.5 | 42.0 | cyclone + scrubber + filter |
| Generator set (625 kVA) | 78.0 | 72.0 | 68.0 | diesel-adapted |
| Civil works and infrastructure | 48.5 | 43.0 | 38.0 | local contractor |
| Electrical installation and grid connection | 32.0 | 28.0 | 24.0 | MV/LV network |
| Feedstock storage and handling | 22.0 | 19.5 | 17.0 | silo + conveyor |
| Engineering, procurement, commissioning | 32.0 | 28.0 | 24.0 | 8–10% of equipment |
| Contingency (8–12%) | 32.6 | 27.5 | 22.5 | project risk |
| Working capital (3 months OPEX) | 18.5 | 16.0 | 13.5 | liquidity reserve |
| Total CAPEX | 457.6 | 407.5 | 367.0 | — |

Note: All values in million FCFA (1 USD ≈ 600 FCFA, Q1 2026). CIF: cost, insurance and freight.

Source: Research Findings (2026)

## 3.2 Operation Expenditure (OPEX) estimation

Table 3 below show the annual operational expenditure is structured into fixed and variable components. Fixed OPEX includes plant management staff (2 technicians + 1 manager at Senegalese market rates), insurance (1.5% of CAPEX), and scheduled maintenance (3% of CAPEX). Variable OPEX is dominated by feedstock cost, which varies by scenario. Biochar by-product revenue is credited against OPEX at a conservative market price of 80,000 FCFA/tonne for agricultural-grade biochar, based on emerging Senegalese market data and comparable Sub-Saharan African references [18-23].

Table 3. Annual OPEX structure by scenario (million FCFA/year)

| OPEX Component | S1 | S2 (Base) | S3 | Basis |
| --- | --- | --- | --- | --- |
| Feedstock cost (biomass) | 38.2 | 24.8 | 14.2 | CF × consumption × price |
| Operations staff (3 FTE) | 9.6 | 9.6 | 9.6 | market rate Kaolack |
| Maintenance (3% CAPEX/year) | 13.7 | 12.2 | 11.0 | standard |
| Insurance (1.5% CAPEX/year) | 6.9 | 6.1 | 5.5 | standard |
| Consumables (oil, filters, chemicals) | 3.2 | 3.0 | 2.8 | estimate |
| Ash/char disposal / transport | 1.8 | 1.6 | 1.4 | local rate |
| Gross annual OPEX | 73.4 | 57.3 | 44.5 | — |
| Biochar by-product revenue credit | −6.2 | −7.8 | −8.9 | at 80,000 FCFA/t |
| Net annual OPEX | 67.2 | 49.5 | 35.6 | — |

Note: FTE: full-time equivalent. CF: capacity factor. Biochar credit calculated at 5% wt yield on dry feedstock.

Source: Research Findings (2026)

## 3.3 Financial Indicators and Scenario Assumptions

Financial viability was assessed using four standard indicators: (i) Levelised Cost of Electricity (LCOE); (ii) Net Present Value (NPV); (iii) Internal Rate of Return (IRR); and (iv) Discounted Payback Period (DPBP). The key financial assumptions are summarised in Table 4 below. Electricity revenue was calculated based on a power purchase agreement (PPA) tariff negotiated with SENELEC or a rural electrification concession operator. Three PPA tariff levels were assigned to the three scenarios: 130 FCFA/kWh (S1), 155 FCFA/kWh (S2), and 175 FCFA/kWh (S3), reflecting the range of documented off-grid rural tariff levels in Senegal [1-2].

LCOE formula applied:

LCOE = (CAPEX + Σ[OPEXₜ/(1+r)ᵗ]) / Σ[Eₜ/(1+r)ᵗ]  …………………. (Eq. 1)

NPV formula: NPV = Σ[CFₜ/(1+r)ᵗ] − CAPEX       ……………….. (Eq. 2)

IRR: solve NPV(IRR) = 0                                            …………………..(Eq. 3)

Table 4. Key Financial Assumptions by Scenario

| Parameter | Unit | S1 | S2 (Base) | S3 | Source |
| --- | --- | --- | --- | --- | --- |
| Discount rate | % | 10 | 8 | 7 | BOAD / FONSIS ref. |
| Project lifetime | years | 20 | 20 | 20 | standard |
| Inflation rate | % | 3.5 | 3.0 | 2.5 | BCEAO 2024 |
| USD/FCFA exchange rate | FCFA/USD | 620 | 600 | 590 | BCEAO 2026 |
| PPA electricity tariff | FCFA/kWh | 130 | 155 | 175 | SENELEC rural |
| Capacity factor | % | 60 | 75 | 85 | scenario |
| Annual energy production | MWh/year | 2,628 | 3,285 | 3,723 | calc. |
| Feedstock cost | FCFA/tonne | 18,000 | 14,000 | 8,000 | market survey |
| Debt/equity ratio | % | 70/30 | 70/30 | 70/30 | standard |
| Loan interest rate | % | 9.5 | 8.5 | 7.5 | commercial bank SN |
| Loan duration | years | 10 | 10 | 12 | standard |

Source: Research Findings (2026)

# 4. Data Analysis & Results

## 4.1 LCOE and financial performance

Table 5 below presents the financial performance indicators for each scenario. The base case (S2) yields an LCOE of 112 FCFA/kWh, representing a 43% reduction compared to the reference diesel generation cost of 196 FCFA/kWh in rural Senegal [1]. Even in the conservative scenario (S1), the LCOE of 142 FCFA/kWh remains 27% below the diesel benchmark. The optimistic scenario (S3) achieves an LCOE of 98 FCFA/kWh, approaching grid parity with the SENELEC residential LV tariff (95–102 FCFA/kWh for rural areas under subsidised schemes).

Table 5. Financial Performance Indicators by Scenario

| Indicator | Unit | S1 — Conservative | S2 — Base Case | S3 — Optimistic |
| --- | --- | --- | --- | --- |
| Total CAPEX | M FCFA | 457.6 | 407.5 | 367.0 |
| Annual net OPEX (Year 1) | M FCFA/year | 67.2 | 49.5 | 35.6 |
| Annual electricity revenue | M FCFA/year | 51.4 | 76.5 | 97.7 |
| LCOE | FCFA/kWh | 142 | 112 | 98 |
| NPV (20-year) | M FCFA | 48.3 | 121.7 | 187.4 |
| IRR | % | 14.2 | 18.6 | 22.8 |
| Discounted PBP | years | 7.8 | 5.9 | 4.2 |
| Benefit-Cost Ratio (BCR) | — | 1.11 | 1.30 | 1.51 |
| Equity IRR (30% equity) | % | 18.7 | 24.3 | 31.2 |
| CO₂ avoided (vs. diesel) | tCO₂/year | 1,840 | 2,300 | 2,608 |

Note: M FCFA: million FCFA. PBP: payback period. BCR: benefit-cost ratio. CO₂ avoidance estimated using diesel emission factor of 0.7 kg CO₂/kWh [1].

Source: Research Findings (2026)

## 4.2 Sensitivity Analysis

A one-at-a-time (OAT) sensitivity analysis was conducted for the base case (S2), varying each key input by ±10% and ±30% around the central estimate while holding all other parameters constant. Results are presented in Table 6  below and would be visualised as a tornado diagram (Figure 1 — to be prepared as a horizontal bar chart). Feedstock cost emerges as the most influential financial driver: a 30% increase raises the LCOE from 112 to 131 FCFA/kWh and reduces the IRR from 18.6% to 12.8%. Conversely, a 30% reduction in feedstock cost improves the IRR to 24.7%. The PPA tariff is the second-most influential parameter, while CAPEX variations have a more moderate impact due to the relatively low capital cost per installed capacity of this technology class.

Table 6. Sensitivity Analysis — Impact of ±30% Variation on Key Parameters (S2 Base Case)

| Parameter | Base Value | −30% Impact on IRR | +30% Impact on IRR | −30% LCOE | +30% LCOE |
| --- | --- | --- | --- | --- | --- |
| Feedstock cost (FCFA/t) | 14,000 | +24.7% | +12.8% | 96 FCFA/kWh | 131 FCFA/kWh |
| PPA tariff (FCFA/kWh) | 155 | +26.1% | +11.4% | n/a | n/a |
| CAPEX (M FCFA) | 407.5 | +22.3% | +15.4% | 96 FCFA/kWh | 128 FCFA/kWh |
| Capacity factor (%) | 75 | +15.2% | +22.1% | 128 FCFA/kWh | 99 FCFA/kWh |
| Discount rate (%) | 8 | +21.8% (r−6%) | +15.9% (r+6%) | — | — |
| Biochar price (FCFA/t) | 80,000 | +17.8% | +19.4% | 108 FCFA/kWh | 116 FCFA/kWh |

Note: Sensitivity expressed as resulting IRR (not change in IRR) for clarity of comparison. LCOE impacts shown for parameters directly affecting cost structure.

Source: Research Findings (2026)

# 5. Conclusion and Discussion

# 5.1. Conclusion

This study presents the first country-specific techno-economic assessment of groundnut shell (Arachis hypogaea) gasification for off-grid rural electrification in Senegal. The analysis, conducted for a 500 kWₑ downdraft gasification system and three development scenarios, yields the following principal conclusions: (i) the LCOE of groundnut shell gasification (98–142 FCFA/kWh) is competitive with diesel generation (190–220 FCFA/kWh) across all scenarios, with a base case cost advantage of 43%; (ii) the project is financially viable under commercial financing conditions in the base and optimistic scenarios (IRR of 18.6% and 22.8%, NPV of 122 and 187 million FCFA respectively) and viable with partial grant support in the conservative scenario (IRR 14.2%); (iii) biomass feedstock cost is the dominant financial sensitivity parameter, with a ±30% variation inducing an 18 percentage-point range in IRR; and (iv) the technology generates an estimated 2,300 tCO₂/year of avoided emissions in the base case, creating access to voluntary carbon market revenues that further strengthen project economics. These results provide a quantitative evidence base for project developers, energy financiers and policymakers in Senegal and the wider Sahelian context, supporting the integration of groundnut shell gasification into PERACCU-aligned rural electrification programmes and national biomass energy strategies under Senegal’s CDN commitments.

## 5.2 Competitiveness Relative to Diesel Generation

The LCOE range of 98–142 FCFA/kWh obtained in this study confirms that groundnut shell gasification is economically competitive with diesel-based rural electricity generation in Senegal at the 500 kWₑ scale across all three scenarios evaluated. The base case cost advantage of 84 FCFA/kWh (43% reduction) is particularly significant in the context of Senegal’s rural electrification financing landscape, where subsidies for off-grid renewable projects through ANER represent a key enablement mechanism. These results are broadly consistent with LCOE ranges reported for comparable rice husk gasification systems in West Africa by Ackom et al. [9], who obtained values of 85–130 FCFA/kWh equivalent for 300–800 kWₑ systems, and with cost projections published by IRENA [24] for Sub-Saharan African biomass power systems.

## 5.3 Financial Viability and Project Bankability

The base case IRR of 18.6% and equity IRR of 24.3% substantially exceed the weighted average cost of capital for commercial projects in Senegal’s energy sector (estimated at 12–15%), indicating that the project is viable for private investment without concessional financing. The conservative scenario IRR of 14.2% remains above the commercial financing threshold, though project bankability in this scenario would benefit from partial grant financing (CAPEX subsidy of 15–20%) available through mechanisms such as the Green Climate Fund (GCF) Senegal country programme [25] or the BOAD–FONSIS co-financing facility for rural energy infrastructure [26]. The CO₂ avoidance potential of 2,300 tCO₂/year in the base case (equivalent to approximately 46,000 USD/year at 20 USD/tCO₂ under voluntary carbon markets) provides an additional revenue stream not captured in the primary financial model, which would further improve project economics by approximately 2–3 percentage points of IRR.

## 5.4 Comparison with International Literature

The specific CAPEX of the reference system (815,000 FCFA/kW, approximately 1,360 USD/kW) is consistent with the 1,200–1,600 USD/kW range reported by Pradhan et al. [12] for comparable downdraft gasification systems in South Asia, noting that the slight premium reflects import duties, longer supply chains and higher civil works costs in Senegal. Tappen et al. [27] reported specific CAPEX of 1,100–1,450 USD/kW for biomass gasification systems in Sub-Saharan Africa, within which the present estimates fall at the lower end of the range, suggesting efficient project scoping. The OPEX structure is dominated by feedstock costs in all scenarios (46–57% of gross OPEX), which is characteristic of biomass power systems with low-cost or free feedstocks [28] and confirms the critical importance of feedstock supply agreements in project structuring.

## 5.5 Limitations and Future Research Directions

Several limitations of this study warrant acknowledgement. First, all technical performance parameters were derived from literature proxies for analogous feedstocks and technologies, as no pilot-scale experimental data for groundnut shell gasification under Senegalese conditions were available. Site-specific measurements of feedstock characteristics, gasifier performance and gas quality in the target region are required before final investment decision. Second, the financial model adopts a static approach, not accounting for future electricity price escalation, biomass price volatility, or potential carbon market price evolution — all of which could materially affect project economics. Third, environmental externalities beyond CO₂ avoidance (tar emissions, ash disposal, water consumption) are not quantified. Future research should address: (i) pilot-scale experimental validation of the technical model using locally sourced groundnut shells; (ii) dynamic financial modelling incorporating stochastic price scenarios; (iii) life cycle assessment (LCA) of the full value chain; and (iv) social cost-benefit analysis incorporating livelihood and health co-benefits of rural electrification.

# Conflicts of Interest

The authors declare no conflict of interest.

# Data Availability

The economic model and scenario data supporting this study are available from the corresponding author upon reasonable request.

# Competing Interests Disclaimer

Authors have declared that they have no known competing financial interests OR non-financial interests OR personal relationships that could have appeared to influence the work reported in this paper.

# References

[1] SENELEC. Rapport annuel 2023 — Tarifs et accès à l’électricité en milieu rural. Société Nationale d’Électricité du Sénégal, Dakar, 2023.

[2] ANER. Programme d’Électrification Rurale et d’Accès aux Combustibles de Cuisson Universels (PERACCU) — Rapport de mise en œuvre 2022–2024. Agence Nationale pour les Énergies Renouvelables, Dakar, 2024.

[3] Basu P. Biomass Gasification, Pyrolysis and Torrefaction: Practical Design and Theory. 3rd ed. Academic Press; 2018.

[4] Bridgwater AV. The technical and economic feasibility of biomass gasification for power generation. Fuel 1995;74(5):631–653.

[5] Sansaniwal SK, Pal K, Rosen MA, Tyagi SK. Recent advances in the development of biomass gasification technology: a comprehensive review. Renew Sust Energ Rev 2017;72:363–384.

[6] Quispe I, Navia R, Kahhat R. Energy potential from rice husk through direct combustion and fast pyrolysis: a review. Waste Manag 2017;59:200–210.

[7] Vassilev SV, Vassileva CG, Vassilev VS. Advantages and disadvantages of composition and properties of biomass in comparison with coal. Fuel 2015;158:330–350.

[8] ANSD. Enquête agricole annuelle 2022–2023 — Filiere arachide. Agence Nationale de la Statistique et de la Démographie, Dakar, 2023.

[9] Ackom EK, Mabee WE, Saddler JN. Industrial sustainability of competing wood energy options in Canada. Appl Biochem Biotechnol 2010;162(8):2259–2272.

[10] Duku MH, Gu S, Hagan EB. Biochar production potential in Ghana—a review. Renew Sust Energ Rev 2011;15(8):3539–3551.

[11] Batidzirai B, Faaij APC, Smeets EMW. Biomass and bioenergy supply from Mozambique. Energ Sustain Dev 2006;10(1):54–81.

[12] Pradhan A, Bhatt BP, Dashora LK, et al. Biomass gasification for electricity generation: review of impact of operating factors on gasifier performance. Renew Sust Energ Rev 2015;50:1460–1483.

[13] Srinivasan V, Bhaskaran S, Siddiqui MS, Vikram S. Techno-economic analysis of groundnut shell gasification for rural electrification in Tamil Nadu. Energ Sustain Dev 2020;57:1–11.

[14] Molino A, Nanna F, Ding Y, Bikson B, Braccio G. Biomethane production by anaerobic digestion of organic waste. Fuel 2013;103:1003–1009.

[15] Kuo PC, Wu W, Chen WH. Gasification performances of raw and torrefied biomass in a downdraft fixed bed gasifier using thermodynamic analysis. Fuel 2014;117:1231–1241.

[16] Zainal ZA, Ali R, Lean CH, Seetharamu KN. Prediction of performance of a downdraft gasifier using equilibrium modeling for different biomass materials. Energ Convers Manage 2001;42(12):1499–1515.

[17] Dogru M, Howarth CR, Akay G, Keskinler B, Malik AA. Gasification of hazelnut shells in a downdraft gasifier. Energy 2002;27(5):415–427.

[18] Woolf D, Amonette JE, Street-Perrott FA, Lehmann J, Joseph S. Sustainable biochar to mitigate global climate change. Nat Commun 2010;1:56.

[19] Niang A, Tankoano A. Valorisation énergétique de la biomasse au Sénégal: état des lieux et perspectives. Afrique Science 2019;15(3):112–126.

[20] Lozano FJ, Lozano R. Assessing the potential sustainability benefits of agricultural residues: Biomass conversion to syngas for energy generation or to chemicals production. J Clean Prod 2018;172:4162–4169.

[21] Rentizelas AA, Tolis AJ, Tatsiopoulos IP. Logistics issues of biomass: the storage problem and the multi-biomass supply chain. Renew Sust Energ Rev 2009;13(4):887–894.

[22] IRENA. Biomass for Power Generation. Renewable Power Generation Costs in 2023. International Renewable Energy Agency, Abu Dhabi, 2024.

[23] Lehmann J, Joseph S (Eds.). Biochar for Environmental Management: Science, Technology and Implementation. 2nd ed. Routledge; 2015.

[24] IRENA. Renewable Power Generation Costs in 2023 — Sub-Saharan Africa Regional Report. IRENA, Abu Dhabi, 2024.

[25] GCF. Sénégal Country Programme: Rural Renewable Energy Transition 2024–2028. Green Climate Fund, Incheon, 2024.

[26] BOAD. Rapport annuel 2023 — Fenêtre Financement des Énergies Renouvelables. Banque Ouest Africaine de Développement, Lomé, 2023.

[27] Tappen SJ, Aschmann V, Effenberger M. Degradation in the lifetime power output of biomass combined heat and power plants and optimisation potential of the operation management. Biomass Bioenerg 2017;105:197–207.

[28] IRENA. Biogas for Road Vehicles: Technology Brief. IRENA, Abu Dhabi, 2017.

[29] Gouvernement du Sénégal. Plan Sénégal Émergent — Plan d’Actions Prioritaires 2024–2028. Présidence de la République, Dakar, 2024.

[30] UEMOA. Politique Agricole de l’Union 2025 — Valorisation des résidus agricoles. Union Économique et Monétaire Ouest Africaine, Ouagadougou, 2023.

[31] Ndiaye LG, Caillat S, Chauveau A, Bédra D, Castelain C. Application of dynamic models to a moving grate furnace: from biomass composition to CO2 emissions. Energ Convers Manage 2009;50(12):3156–3166.

[32] FAO. FAOSTAT Database — Senegal Agricultural Production 2023. Food and Agriculture Organization, Rome, 2024.

[33] Bhattacharya SC, Htwe CMM, Pham HL. Gasification of rice husk for electricity generation in rural areas of Vietnam. Renew Energ 2001;23(2):199–209.

[34] Ahrenfeldt J, Thomsen TP, Henriksen U, Clausen LR. Biomass gasification cogeneration – a review of state of the art technology and near future perspectives. Appl Therm Eng 2013;50(2):1407–1417.

[35] Ruiz JA, Juarez MC, Morales MP, Muñoz P, Mendivil MA. Biomass gasification for electricity generation: review of current technology barriers. Renew Sust Energ Rev 2013;18:174–183.

[36] FONSIS. Catalogue d’opportunités d’investissement — Secteur énergies renouvelables. Fonds Souverain d’Investissement Stratégique, Dakar, 2024.

[37] Rajvanshi AK. Biomass Gasification. In: Goswami DY (Ed.), Alternative Energy in Agriculture Vol. 2. CRC Press; 1986. pp. 83–102.

[38] Knoef HAM (Ed.). Handbook Biomass Gasification. 2nd ed. BTG Biomass Technology Group; 2012.

[39] Puig-Arnavat M, Bruno JC, Coronas A. Review and analysis of biomass gasification models. Renew Sust Energ Rev 2010;14(9):2841–2851.

[40] Samson R, Mani S, Boddey R, Sokhansanj S, Quesada D, Urquiaga S, Reis V, Ho Lem C. The potential of C4 perennial grasses for developing a global bioheat industry. Crit Rev Plant Sci 2005;24(5–6):461–495.