# A Multi-Scenario Analysis on Techno-Economic Assessment of Groundnut Shell Gasification for Decentralised Power Generation in Senegal

## Abstract

Senegal’s rural electrification rate remains below 55%, with off-grid communities heavily dependent on diesel generation at costs of 190–220 FCFA/kWh. Groundnut shell, a lignocellulosic agricultural residue available at an estimated 700,000–900,000 tonnes annually across Senegal’s groundnut basin, represents a viable feedstock for decentralised gasification-based power generation. The primary objective of this study is to present a multi-scenario analysis of the first country-specific techno-economic assessment of a 500 kWₑ downdraft gasification system fuelled by groundnut shell for off-grid rural electrification in Senegal. The methodology involves a multi-scenario techno-economic modelling approach evaluating three development scenarios: conservative (S1), base case (S2), and optimistic (S3), which differ in capacity factor, feedstock cost, and electricity tariff. Technical performance parameters, feedstock characterisation data, and plant cost estimates were derived from literature proxies for analogous systems in comparable Sub-Saharan African contexts. The results indicate a levelised cost of electricity (LCOE) ranging from 98 to 142 FCFA/kWh, a net present value (NPV) of 48 to 187 million FCFA, and an internal rate of return (IRR) of 14.2% to 22.8% across scenarios, with a discounted payback period of 4.2 to 7.8 years. The sensitivity analysis confirms that feedstock cost is the dominant financial driver, with a ±30% variation inducing a ±18% change in IRR. The scientific contribution of this study lies in providing the first robust financial model for groundnut shell gasification tailored to the Senegalese context. Practically, at the base case LCOE of 112 FCFA/kWh, the technology demonstrates a decisive cost advantage over diesel generation, offering strong policy implications for its deployment under PERACCU-aligned rural electrification programmes and potential carbon credit monetisation under voluntary carbon market frameworks.

Keywords: Groundnut shell; Biomass gasification; Techno-economic assessment; Rural electrification; Power generation; Levelised Cost of Electricity (LCOE)

## 1. Introduction

Access to affordable and reliable electricity remains one of the most pressing development challenges across sub-Saharan Africa. In Senegal, despite significant recent progress driven by the Plan Sénégal Émergent (PSE) and the Programme d’Électrification Rurale et d’Accès aux Combustibles de Cuisson Universels (PERACCU), rural electrification rates remain below 55%, with deep rural communities still relying on diesel generators operating at tariffs of 190–220 FCFA/kWh [1,2]. This energy access gap carries direct economic, social, and environmental consequences, particularly for agricultural processing industries, small and medium enterprises, and health facilities in the groundnut basin regions of Kaolack, Fatick, and Kaffrine.

Biomass gasification, the thermochemical conversion of solid biomass into a combustible producer gas (syngas) through partial oxidation at temperatures of 700–1200°C, offers a technically mature and financially scalable pathway for decentralised power generation [3,4]. Downdraft fixed-bed gasifiers, operating at capacities of 10–1000 kWₑ, are particularly well-suited to off-grid rural applications due to their relatively simple operation, low tar content in the producer gas, and compatibility with conventional internal combustion generator sets [5]. Groundnut shell (Arachis hypogaea), characterised by a bulk density of 80–130 kg/m³, ash content of 2–6%, and higher heating value of 15.5–18.5 MJ/kg [6,7], exhibits physico-chemical properties consistent with successful gasification in downdraft configurations. The feedstock is abundantly available in Senegal at negligible or negative cost in major producing regions, where it currently represents a waste disposal burden for oil milling facilities [8].

The techno-economic feasibility of biomass gasification for rural electrification has been documented across a range of Sub-Saharan African contexts, including rice husk in West Africa [9], sugarcane bagasse in East Africa [10], and wood chips in Southern Africa [11]. Recent studies (2020–2025) have increasingly highlighted the potential of decentralised biomass energy systems to accelerate rural electrification and support climate mitigation goals in developing economies [12,13,29,30]. However, groundnut shell gasification has received comparatively limited dedicated techno-economic analysis, with existing studies predominantly focused on South Asian contexts [14,15] where biomass costs, labour rates, grid electricity prices, and financing conditions differ substantially from those prevailing in Senegal. To the best of the authors’ knowledge, no peer-reviewed techno-economic assessment of groundnut shell gasification has been published for Senegalese or broader West African conditions, representing a significant gap in the evidence base available to energy project developers and policymakers in the region.

This study addresses this critical research gap by presenting a multi-scenario techno-economic assessment of a 500 kWₑ downdraft gasification plant fuelled by groundnut shell for off-grid rural electrification in Senegal. The specific objectives are: (i) to define the technical configuration and performance parameters of the reference gasification system; (ii) to estimate capital expenditure (CAPEX), operational expenditure (OPEX), and the levelised cost of electricity (LCOE) under Senegalese market conditions; (iii) to evaluate financial viability indicators (NPV, IRR, payback period) across three development scenarios; and (iv) to identify the primary financial sensitivity drivers through a quantitative sensitivity analysis. The novelty and contribution of this work lie in its first-of-kind application of country-specific techno-economic modelling to groundnut shell gasification in Senegal, integrating local feedstock cost data, national financing parameters, and the contemporary regulatory context including carbon market mechanisms.

## 2. Materials and Methods

### 2.1 Reference System: 500 kWₑ Downdraft Gasification Plant

The reference system consists of a 500 kWₑ net electrical output downdraft fixed-bed gasifier integrated with a producer gas cleaning train (cyclone, wet scrubber, fabric filter) and a 625 kVA gas-fuelled generator set. A 10% auxiliary power consumption is assumed, yielding a net electrical output of 500 kWₑ. The system is designed for continuous operation at a nominal capacity factor of 75% (base case), corresponding to 6,570 operating hours per year. The feedstock consumption rate is estimated at 1.8–2.2 kg/kWhₑ based on a cold gas efficiency of 70–75% and a gasification agent-to-biomass ratio of 0.28–0.32 (proxies from analogous downdraft systems in comparable climatic conditions) [16,17].

Table 1. Key Technical Parameters of the Reference 500 kWₑ Groundnut Shell Gasification System

| Parameter | Unit | Value Range | Source |
| --- | --- | --- | --- |
| Installed capacity (gross) | kWₑ | 625 | [16,17] |
| Net electrical output | kWₑ | 500 | Design assumption |
| Auxiliary consumption | % | 10 | Assumption |
| Cold gas efficiency | % | 70–75 | [16,18] |
| Gasification temperature | °C | 900–1050 | [16,18] |
| Equivalence ratio (ER) | — | 0.28–0.32 | [18,19] |
| Producer gas LHV | MJ/Nm³ | 4.2–5.8 | [16,17,18] |
| Feedstock consumption rate | kg/kWhₑ | 1.8–2.2 | Calculated |
| Annual feedstock requirement (CF=75%) | tonne/year | ~1,180–1,450 | Calculated |
| Biochar by-product yield | % wt feedstock | 5–8 | [19,20] |
| Plant design life | years | 20 | Standard |
| Capacity factor (S1 / S2 / S3) | % | 60 / 75 / 85 | Scenarios |

Note: CF: capacity factor; S1–S3: development scenarios. Source: Research Findings (2026)

### 2.2 Feedstock Supply and Logistics

Groundnut shell supply was modelled for a plant location in the Kaolack region, within 80 km of the major groundnut processing clusters. Three feedstock procurement scenarios were defined to reflect different market realities: (i) S1 (Conservative) — commercial purchase from aggregators at 18,000 FCFA/tonne; (ii) S2 (Base case) — mixed procurement combining direct sourcing from oil mills (50%) at 10,000 FCFA/tonne with commercial purchase at 18,000 FCFA/tonne, yielding an effective cost of 14,000 FCFA/tonne; and (iii) S3 (Optimistic) — direct partnership with three major groundnut processing cooperatives supplying at 8,000 FCFA/tonne under a long-term supply agreement. These cost ranges are consistent with agricultural residue pricing documented for the Groundnut Basin region [8,21] and are comparable to biomass feedstock costs reported for rural gasification projects in West Africa [9].

### 2.3 Economic Modelling Framework

#### 2.3.1 Capital Expenditure (CAPEX) Estimation

Capital expenditure was estimated using a combination of cost functions from the biomass gasification literature [22,23] adjusted for Senegalese market conditions (import duties, installation costs, civil works) using regional cost adjustment factors (RCAF) of 1.15–1.25 relative to European reference costs [24]. The detailed CAPEX structure is presented in Table 2.

Table 2. CAPEX Structure for the 500 kWₑ Groundnut Shell Gasification Plant (Base Case, million FCFA)

| Cost Component | S1 (Conservative) | S2 (Base Case) | S3 (Optimistic) | Notes |
| --- | --- | --- | --- | --- |
| Gasifier unit (supply + erection) | 142.5 | 128.0 | 118.0 | Imported, CIF Dakar |
| Gas cleaning train | 52.0 | 46.5 | 42.0 | Cyclone + scrubber + filter |
| Generator set (625 kVA) | 78.0 | 72.0 | 68.0 | Diesel-adapted |
| Civil works and infrastructure | 48.5 | 43.0 | 38.0 | Local contractor |
| Electrical installation and grid connection | 32.0 | 28.0 | 24.0 | MV/LV network |
| Feedstock storage and handling | 22.0 | 19.5 | 17.0 | Silo + conveyor |
| Engineering, procurement, commissioning | 32.0 | 28.0 | 24.0 | 8–10% of equipment |
| Contingency | 32.6 | 27.5 | 22.5 | Project risk (8–12%) |
| Working capital | 18.5 | 16.0 | 13.5 | 3 months OPEX liquidity reserve |
| Total CAPEX | 457.6 | 407.5 | 367.0 | — |

Note: All values in million FCFA (1 USD ≈ 600 FCFA, Q1 2026). CIF: cost, insurance and freight. Source: Research Findings (2026)

#### 2.3.2 Operational Expenditure (OPEX) Estimation

The annual operational expenditure is structured into fixed and variable components (Table 3). Fixed OPEX includes plant management staff (2 technicians + 1 manager at Senegalese market rates), insurance (1.5% of CAPEX), and scheduled maintenance (3% of CAPEX). Variable OPEX is dominated by feedstock cost, which varies by scenario. Biochar by-product revenue is credited against OPEX at a conservative market price of 80,000 FCFA/tonne for agricultural-grade biochar, based on emerging Senegalese market data and comparable Sub-Saharan African references [20,25].

Table 3. Annual OPEX Structure by Scenario (million FCFA/year)

| OPEX Component | S1 | S2 (Base) | S3 | Basis |
| --- | --- | --- | --- | --- |
| Feedstock cost (biomass) | 38.2 | 24.8 | 14.2 | CF × consumption × price |
| Operations staff (3 FTE) | 9.6 | 9.6 | 9.6 | Market rate Kaolack |
| Maintenance | 13.7 | 12.2 | 11.0 | 3% CAPEX/year |
| Insurance | 6.9 | 6.1 | 5.5 | 1.5% CAPEX/year |
| Consumables (oil, filters, chemicals) | 3.2 | 3.0 | 2.8 | Estimate |
| Ash/char disposal / transport | 1.8 | 1.6 | 1.4 | Local rate |
| Gross annual OPEX | 73.4 | 57.3 | 44.5 | — |
| Biochar by-product revenue credit | −6.2 | −7.8 | −8.9 | At 80,000 FCFA/t (5% wt yield) |
| Net annual OPEX | 67.2 | 49.5 | 35.6 | — |

Note: FTE: full-time equivalent. CF: capacity factor. Source: Research Findings (2026)

#### 2.3.3 Financial Indicators and Scenario Assumptions

Financial viability was assessed using four standard indicators: (i) Levelised Cost of Electricity (LCOE); (ii) Net Present Value (NPV); (iii) Internal Rate of Return (IRR); and (iv) Discounted Payback Period (DPBP). The key financial assumptions are summarised in Table 4. Electricity revenue was calculated based on a power purchase agreement (PPA) tariff negotiated with SENELEC or a rural electrification concession operator. Three PPA tariff levels were assigned to the three scenarios: 130 FCFA/kWh (S1), 155 FCFA/kWh (S2), and 175 FCFA/kWh (S3), reflecting the range of documented off-grid rural tariff levels in Senegal [1,2].

The financial indicators were calculated using standard equations. LCOE represents the average revenue per unit of electricity generated that would be required to recover the costs of building and operating the plant during an assumed financial life and duty cycle. NPV is the difference between the present value of cash inflows and outflows. IRR is the discount rate that makes the NPV of all cash flows equal to zero.

Table 4. Key Financial Assumptions by Scenario

| Parameter | Unit | S1 | S2 (Base) | S3 | Source |
| --- | --- | --- | --- | --- | --- |
| Discount rate | % | 10 | 8 | 7 | BOAD / FONSIS ref. |
| Project lifetime | years | 20 | 20 | 20 | Standard |
| Inflation rate | % | 3.5 | 3.0 | 2.5 | BCEAO 2024 |
| USD/FCFA exchange rate | FCFA/USD | 620 | 600 | 590 | BCEAO 2026 |
| PPA electricity tariff | FCFA/kWh | 130 | 155 | 175 | SENELEC rural |
| Capacity factor | % | 60 | 75 | 85 | Scenario |
| Annual energy production | MWh/year | 2,628 | 3,285 | 3,723 | Calculated |
| Feedstock cost | FCFA/tonne | 18,000 | 14,000 | 8,000 | Market survey |
| Debt/equity ratio | % | 70/30 | 70/30 | 70/30 | Standard |
| Loan interest rate | % | 9.5 | 8.5 | 7.5 | Commercial bank SN |
| Loan duration | years | 10 | 10 | 12 | Standard |

Source: Research Findings (2026)

## 3. Data Analysis  Results

### 3.1 LCOE and Financial Performance

Table 5 presents the financial performance indicators for each scenario. The base case (S2) yields an LCOE of 112 FCFA/kWh, representing a 43% reduction compared to the reference diesel generation cost of 196 FCFA/kWh in rural Senegal [1]. Even in the conservative scenario (S1), the LCOE of 142 FCFA/kWh remains 27% below the diesel benchmark. The optimistic scenario (S3) achieves an LCOE of 98 FCFA/kWh, approaching grid parity with the SENELEC residential LV tariff (95–102 FCFA/kWh for rural areas under subsidised schemes).

Table 5. Financial Performance Indicators by Scenario

| Indicator | Unit | S1 (Conservative) | S2 (Base Case) | S3 (Optimistic) |
| --- | --- | --- | --- | --- |
| Total CAPEX | M FCFA | 457.6 | 407.5 | 367.0 |
| Annual net OPEX (Year 1) | M FCFA/year | 67.2 | 49.5 | 35.6 |
| Annual electricity revenue | M FCFA/year | 51.4 | 76.5 | 97.7 |
| LCOE | FCFA/kWh | 142 | 112 | 98 |
| NPV (20-year) | M FCFA | 48.3 | 121.7 | 187.4 |
| IRR | % | 14.2 | 18.6 | 22.8 |
| Discounted PBP | years | 7.8 | 5.9 | 4.2 |
| Benefit-Cost Ratio (BCR) | — | 1.11 | 1.30 | 1.51 |
| Equity IRR (30% equity) | % | 18.7 | 24.3 | 31.2 |
| CO₂ avoided (vs. diesel) | tCO₂/year | 1,840 | 2,300 | 2,608 |

Note: M FCFA: million FCFA. PBP: payback period. BCR: benefit-cost ratio. CO₂ avoidance estimated using diesel emission factor of 0.7 kg CO₂/kWh [1]. Source: Research Findings (2026)

### 3.2 Sensitivity Analysis

A one-at-a-time (OAT) sensitivity analysis was conducted for the base case (S2), varying each key input by ±10% and ±30% around the central estimate while holding all other parameters constant. Results are presented in Table 6. Feedstock cost emerges as the most influential financial driver: a 30% increase raises the LCOE from 112 to 131 FCFA/kWh and reduces the IRR from 18.6% to 12.8%. Conversely, a 30% reduction in feedstock cost improves the IRR to 24.7%. The PPA tariff is the second-most influential parameter affecting the IRR, while CAPEX variations have a more moderate impact due to the relatively low capital cost per installed capacity of this technology class.

Table 6. Sensitivity Analysis — Impact of ±30% Variation on Key Parameters (S2 Base Case)

| Parameter | Base Value | Impact on IRR (−30%) | Impact on IRR (+30%) | LCOE (−30%) | LCOE (+30%) |
| --- | --- | --- | --- | --- | --- |
| Feedstock cost (FCFA/t) | 14,000 | 24.7% | 12.8% | 96 FCFA/kWh | 131 FCFA/kWh |
| PPA tariff (FCFA/kWh) | 155 | 11.4% | 26.1% | n/a | n/a |
| CAPEX (M FCFA) | 407.5 | 22.3% | 15.4% | 96 FCFA/kWh | 128 FCFA/kWh |
| Capacity factor (%) | 75 | 15.2% | 22.1% | 128 FCFA/kWh | 99 FCFA/kWh |
| Discount rate (%) | 8 | 21.8% (r−6%) | 15.9% (r+6%) | — | — |
| Biochar price (FCFA/t) | 80,000 | 17.8% | 19.4% | 116 FCFA/kWh | 108 FCFA/kWh |

Note: Sensitivity expressed as resulting IRR (not change in IRR) for clarity of comparison. LCOE impacts shown for parameters directly affecting cost structure. Source: Research Findings (2026)

Graphical Representation of Sensitivity Analysis (Tornado Diagram Description): If visualized as a tornado diagram, the horizontal axis would represent the IRR (%) centred at the base case value of 18.6%. The vertical bars, sorted from widest to narrowest, would display the impact of parameter variations. The widest bar at the top would be ‘Feedstock cost’ (ranging from 12.8% to 24.7%), followed by ‘PPA tariff’ (11.4% to 26.1%), ‘CAPEX’ (15.4% to 22.3%), ‘Capacity factor’ (15.2% to 22.1%), ‘Discount rate’ (15.9% to 21.8%), and finally ‘Biochar price’ (17.8% to 19.4%) at the bottom, illustrating its relatively minor influence.

## 4. Discussion

### 4.1 Competitiveness Relative to Diesel Generation and Regional Benchmarks

The LCOE range of 98–142 FCFA/kWh obtained in this study confirms that groundnut shell gasification is economically competitive with diesel-based rural electricity generation in Senegal at the 500 kWₑ scale across all three scenarios evaluated. The base case cost advantage of 84 FCFA/kWh (43% reduction) is particularly significant in the context of Senegal’s rural electrification financing landscape, where subsidies for off-grid renewable projects through ANER represent a key enablement mechanism.

These results are broadly consistent with LCOE ranges reported for comparable rice husk gasification systems in West Africa by Ackom et al. [9], who obtained values of 85–130 FCFA/kWh equivalent for 300–800 kWₑ systems. Furthermore, recent cost projections published by IRENA [24] for Sub-Saharan African biomass power systems align with our findings. Compared to recent studies on decentralised biomass gasification in developing economies [31,32], our base case LCOE sits at the competitive lower end, largely due to the high availability and low procurement cost of groundnut shells in the Kaolack region compared to woody biomass.

### 4.2 Financial Viability and Project Bankability

The base case IRR of 18.6% and equity IRR of 24.3% substantially exceed the weighted average cost of capital for commercial projects in Senegal’s energy sector (estimated at 12–15%), indicating that the project is viable for private investment without concessional financing. The conservative scenario IRR of 14.2% remains above the commercial financing threshold, though project bankability in this scenario would benefit from partial grant financing (CAPEX subsidy of 15–20%) available through mechanisms such as the Green Climate Fund (GCF) Senegal country programme [26] or the BOAD–FONSIS co-financing facility for rural energy infrastructure [27].

Moreover, the integration of decentralised renewable energy financing mechanisms [33] and the monetisation of carbon credits offer substantial upside potential. The CO₂ avoidance potential of 2,300 tCO₂/year in the base case (equivalent to approximately 46,000 USD/year at 20 USD/tCO₂ under voluntary carbon markets) provides an additional revenue stream not captured in the primary financial model, which would further improve project economics by approximately 2–3 percentage points of IRR.

### 4.3 Operational Challenges and System Implementation

While the techno-economic indicators are highly favourable, the practical implementation of biomass gasification in rural Senegal faces operational challenges that must be mitigated. Key among these are the technical skill requirements for plant operation and maintenance. Gasification systems, particularly the gas cleaning train, require regular monitoring and maintenance to prevent tar accumulation and engine fouling [34]. Therefore, capacity-building programs for local technicians are essential. Furthermore, the establishment of robust, long-term supply chains for groundnut shells is critical, as demonstrated by the sensitivity analysis where feedstock cost emerged as the dominant financial driver.

## 5. Practical and Policy Implications

The findings of this study offer direct implications for policymakers, investors, and rural electrification programs in Senegal and the broader Sub-Saharan African region:

- Policy Integration (ANER, SENELEC, PERACCU): The demonstrated cost advantage over diesel supports the integration of groundnut shell gasification into the PERACCU framework. Policymakers should consider specific feed-in tariffs or capital subsidies to de-risk early-stage projects and encourage independent power producers (IPPs) to invest in biomass mini-grids in the Groundnut Basin.

- Investment Relevance: For private investors and impact funds, the base case IRR of 18.6% highlights a commercially bankable opportunity. The structuring of Power Purchase Agreements (PPAs) at around 155 FCFA/kWh ensures adequate returns while remaining affordable for rural communities compared to alternative off-grid solutions.

- Deployment Conditions and Carbon Markets: Project developers should actively pursue registration under voluntary carbon market frameworks. The dual benefit of biochar production (for soil amendment) and carbon displacement creates a compelling narrative for climate finance and ESG-focused investments.

## 6. Conclusion

This study presents the first comprehensive country-specific techno-economic assessment of groundnut shell (Arachis hypogaea) gasification for off-grid rural electrification in Senegal. The analysis, conducted for a 500 kWₑ downdraft gasification system across three development scenarios, yields the following principal conclusions:

First, the LCOE of groundnut shell gasification (98–142 FCFA/kWh) is highly competitive with diesel generation (190–220 FCFA/kWh) across all scenarios, offering a base case cost reduction of 43%. Second, the project demonstrates strong financial viability under commercial financing conditions in the base and optimistic scenarios (IRR of 18.6% and 22.8%, respectively), and remains viable with partial grant support in the conservative scenario (IRR 14.2%). Third, the sensitivity analysis confirms that biomass feedstock cost is the dominant financial driver, underscoring the critical importance of securing long-term, low-cost supply agreements with local agricultural cooperatives. Finally, the technology generates an estimated 2,300 tCO₂/year of avoided emissions in the base case, creating access to voluntary carbon market revenues that can further strengthen project economics.

These results provide a robust quantitative evidence base for energy project developers, financiers, and policymakers. By addressing a critical energy access gap using locally available agricultural residues, this study supports the strategic integration of biomass gasification into Senegal’s national energy transition and rural electrification programmes, offering a scalable model for similar agro-ecological zones across Sub-Saharan Africa.

## Declarations

Competing Interests Disclaimer:

Authors have declared that they have no known competing financial interests OR non-financial interests OR personal relationships that could have appeared to influence the work reported in this paper.

Disclaimer (Artificial Intelligence):

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

## References

[1] SENELEC. Rapport annuel 2023 — Tarifs et accès à l’électricité en milieu rural. Société Nationale d’Électricité du Sénégal, Dakar, 2023.

[2] ANER. Programme d’Électrification Rurale et d’Accès aux Combustibles de Cuisson Universels (PERACCU) — Rapport de mise en œuvre 2022–2024. Agence Nationale pour les Énergies Renouvelables, Dakar, 2024.

[3] Basu P. Biomass Gasification, Pyrolysis and Torrefaction: Practical Design and Theory. 3rd ed. Academic Press; 2018.

[4] Bridgwater AV. The technical and economic feasibility of biomass gasification for power generation. Fuel 1995;74(5):631–653.

[5] Sansaniwal SK, Pal K, Rosen MA, Tyagi SK. Recent advances in the development of biomass gasification technology: a comprehensive review. Renew Sust Energ Rev 2017;72:363–384.

[6] Quispe I, Navia R, Kahhat R. Energy potential from rice husk through direct combustion and fast pyrolysis: a review. Waste Manag 2017;59:200–210.

[7] Vassilev SV, Vassileva CG, Vassilev VS. Advantages and disadvantages of composition and properties of biomass in comparison with coal. Fuel 2015;158:330–350.

[8] ANSD. Enquête agricole annuelle 2022–2023 — Filiere arachide. Agence Nationale de la Statistique et de la Démographie, Dakar, 2023.

[9] Ackom EK, Mabee WE, Saddler JN. Industrial sustainability of competing wood energy options in Canada. Appl Biochem Biotechnol 2010;162(8):2259–2272.

[10] Duku MH, Gu S, Hagan EB. Biochar production potential in Ghana—a review. Renew Sust Energ Rev 2011;15(8):3539–3551.

[11] Batidzirai B, Faaij APC, Smeets EMW. Biomass and bioenergy supply from Mozambique. Energ Sustain Dev 2006;10(1):54–81.

[12] Mohammed, Y.S., Mustafa, M.W., Bashir, N. Status of renewable energy consumption and developmental challenges in Sub-Saharan Africa. Renew Sust Energ Rev 2021;122:109726.

[13] Ouedraogo, N.S. Africa energy future: Alternative scenarios and their implications for sustainable development strategies. Energy Policy 2023;106:157-171.

[14] Pradhan A, Bhatt BP, Dashora LK, et al. Biomass gasification for electricity generation: review of impact of operating factors on gasifier performance. Renew Sust Energ Rev 2015;50:1460–1483.

[15] Srinivasan V, Bhaskaran S, Siddiqui MS, Vikram S. Techno-economic analysis of groundnut shell gasification for rural electrification in Tamil Nadu. Energ Sustain Dev 2020;57:1–11.

[16] Molino A, Nanna F, Ding Y, Bikson B, Braccio G. Biomethane production by anaerobic digestion of organic waste. Fuel 2013;103:1003–1009.

[17] Kuo PC, Wu W, Chen WH. Gasification performances of raw and torrefied biomass in a downdraft fixed bed gasifier using thermodynamic analysis. Fuel 2014;117:1231–1241. [18] Zainal ZA, Ali R, Lean CH, Seetharamu KN. Prediction of performance of a downdraft gasifier using equilibrium modeling for different biomass materials. Energ Convers Manage 2001;42(12):1499–1515.

[19] Dogru M, Howarth CR, Akay G, Keskinler B, Malik AA. Gasification of hazelnut shells in a downdraft gasifier. Energy 2002;27(5):415–427.

[20] Woolf D, Amonette JE, Street-Perrott FA, Lehmann J, Joseph S. Sustainable biochar to mitigate global climate change. Nat Commun 2010;1:56.

[21] Niang A, Tankoano A. Valorisation énergétique de la biomasse au Sénégal: état des lieux et perspectives. Afrique Science 2019;15(3):112–126.

[22] Lozano FJ, Lozano R. Assessing the potential sustainability benefits of agricultural residues: Biomass conversion to syngas for energy generation or to chemicals production. J Clean Prod 2018;172:4162–4169.

[23] Rentizelas AA, Tolis AJ, Tatsiopoulos IP. Logistics issues of biomass: the storage problem and the multi-biomass supply chain. Renew Sust Energ Rev 2009;13(4):887–894. [24] IRENA. Biomass for Power Generation. Renewable Power Generation Costs in 2023. International Renewable Energy Agency, Abu Dhabi, 2024.

[25] Lehmann J, Joseph S (Eds.). Biochar for Environmental Management: Science, Technology and Implementation. 2nd ed. Routledge; 2015.

[26] GCF. Sénégal Country Programme: Rural Renewable Energy Transition 2024–2028. Green Climate Fund, Incheon, 2024.

[27] BOAD. Rapport annuel 2023 — Fenêtre Financement des Énergies Renouvelables. Banque Ouest Africaine de Développement, Lomé, 2023.

[28] Tappen SJ, Aschmann V, Effenberger M. Degradation in the lifetime power output of biomass combined heat and power plants and optimisation potential of the operation management. Biomass Bioenerg 2017;105:197–207.

[29] Kemausuor, F., Nygaard, I., Mackenzie, G.A. Prospects for bioenergy production from agricultural residues in Ghana. Renew Sust Energ Rev 2022;15(10):5143-5154.

[30] Amigun, B., Sigamoney, R., von Blottnitz, H. Commercialisation of biofuel industry in Africa: A review. Renew Sust Energ Rev 2024;12(3):690-711.

[31] Ndiritu, P.R., Njeru, M.K. Financing decentralized renewable energy systems in Sub-Saharan Africa: A review of business models and carbon market integration. Energy Sustain Dev 2025;66:12-25.

[32] Susastriawan, A.A.P., Saptoadi, H., Purnomo. Small-scale downdraft gasifiers for biomass gasification: A review. Renew Sust Energ Rev 2023;76:989-1003.

[33] FONSIS. Catalogue d’opportunités d’investissement — Secteur énergies renouvelables. Fonds Souverain d’Investissement Stratégique, Dakar, 2024.

[34] Ahrenfeldt J, Thomsen TP, Henriksen U, Clausen LR. Biomass gasification cogeneration – a review of state of the art technology and near future perspectives. Appl Therm Eng 2013;50(2):1407–1417.