Integrated Physicochemical and Microbiological Assessment of Shallow water in Agbangnizoun, Southern Benin

Abstract

Background and Aim: Safe drinking water remains a major challenge in rural sub-Saharan Africa, where people rely on shallow groundwater sources. Groundwater quality is influenced by geology, recharge, land use, and human activities, affecting its vulnerability to contamination. This study quantitatively and qualitatively assessed the physicochemical and bacteriological quality of shallow well water (17 to 45 m total depth) in the commune of Agbangnizoun and evaluated its spatial variability using an integrated Pollution Index.

Study Design: This was a dry-season field sampling survey of shallow wells used for domestic water supply in Agbangnizoun."

Place and Duration of Study: The study was carried out in the commune of Agbangnizoun, southern Benin, and sampling was performed during the dry season (December 2023 to March 2024).

Methodology: Twenty-seven shallow wells were sampled and analyzed for pH, electrical conductivity, major ions (Ca²⁺, Mg²⁺, Na⁺, K⁺, HCO₃⁻, Cl⁻, SO₄²⁻, NO₃⁻), total hardness, and fecal indicator bacteria using standard physicochemical and microbiological methods. A Pollution Index based on standardized parameter scores was calculated, and heatmap analysis of the index scores was used to visualize spatial variability and identify dominant contamination drivers.

Results: The groundwater showed predominantly acidic conditions, with a mean pH of 5.86, and low to moderate mineralization dominated by calcium (Ca²⁺) and magnesium (Mg²⁺). Nitrate (NO₃⁻) concentrations exceeded 25 mg/L in most wells, indicating significant anthropogenic influence. All samples were contaminated with fecal indicator bacteria, with Escherichia coli concentrations ranging from 20 to 290 CFU/100 mL. The Pollution Index revealed strong spatial heterogeneity, with wells W6–W9 mainly affected by elevated sulfate (SO₄²⁻) and nitrate (NO₃⁻) levels, while well W26 showed dominant bacteriological contamination.

Conclusion: Shallow well water in Agbangnizoun is highly vulnerable to both chemical and bacteriological pollution and is unsuitable for direct consumption without treatment. The Pollution Index, derived from standardized parameters and visualized by heatmap analysis, effectively identifies pollution hotspots and supports groundwater management in rural settings.

Keywords: Well water quality; Groundwater pollution; Fecal contamination; Nitrates; Pollution Index; Agbangnizoun, Southern Benin

- INTRODUCTION

Access to safe drinking water remains a major public health challenge in many rural areas of sub-Saharan Africa, where populations largely depend on shallow groundwater resources such as hand-dug wells for domestic supply(Lapworth et al. 2017; Liddle et al. 2015). In Benin, despite national efforts to improve access to improved water sources, a substantial proportion of rural households continue to rely on traditional wells that are often poorly protected and highly vulnerable to contamination(Hounsounou et al. 2017; VODOUNOU and DENONSI 2018). These groundwater resources are exposed to both natural hydrogeochemical processes and increasing anthropogenic pressures associated with population growth, agricultural intensification, and inadequate sanitation infrastructure(Zhang et al. 2024)

The quality of groundwater in shallow to moderately deep aquifers is controlled by complex interactions between geological formations, recharge conditions, land use, and human activities(Yuan et al. 2023). In crystalline basement settings, common in southern Benin, groundwater chemistry is often characterized by low to moderate mineralization, acidic pH, and dominance of calcium–magnesium bicarbonate facies resulting from the weathering of silicate and carbonate minerals(Egbueri et al. 2025; Tossou et al. 2017). However, the shallow depth of these aquifers and their limited natural protection make them particularly sensitive to surface-derived pollutants, including nitrates from agricultural inputs and fecal microorganisms originating from latrines, livestock, and domestic wastewater(Wamsley 2014; Ward 2020).

Nitrate contamination of groundwater has emerged as a widespread issue in rural African settings and is commonly used as an indicator of anthropogenic influence in aquifers(Elisante and Muzuka 2017). Elevated nitrate levels in drinking water are associated with adverse health effects, notably methemoglobinemia in infants and potential long-term gastrointestinal disorders(Brender 2020; Chaudhary et al. 2025). In parallel, microbiological contamination of groundwater represents a critical health concern, as the presence of fecal indicator bacteria such as Escherichia coli, total coliforms, and fecal streptococci signals the possible occurrence of pathogenic microorganisms and an increased risk of waterborne diseases(Oon et al. 2023)

Previous studies across Benin have documented chemical degradation of well water quality involving elevated nitrates (NO₃⁻), nitrites (NO₂⁻) alongside bacteriological contamination by Escherichia coli and fecal streptococci(Dovonou et al. 2017). These contaminations arise from both geogenic sources (mineral weathering contributing Ca²⁺, Mg²⁺, HCO₃⁻) and anthropogenic inputs (fertilizers, wastewater, latrines); and often linked to insufficient protection of water points, proximity to sanitation facilities (Marc et al. 2017) . However, these studies are generally site-specific, and data remain scarce for several rural communes. Moreover, integrated assessments combining physicochemical and microbiological parameters into a unified spatial framework remain limited for rural communes in southern Benin, including Agbangnizoun.

In this context, the present study aims to (i) assess the physicochemical and bacteriological quality of well water used by local populations in the commune of Agbangnizoun, (ii) identify the main hydrochemical characteristics and infer potential contamination sources, including diffuse agricultural inputs and localized anthropogenic sources such as sanitation facilities, and (iii) evaluate the spatial heterogeneity of groundwater pollution through the development of a relative Pollution Index (PI) based on standardized parameters. By combining conventional hydrochemical analysis, microbiological indicators, and multivariate visualization tools, this work provides a comprehensive assessment of groundwater quality and contributes to improving local water resource management and public health protection strategies.

- MATERIALS AND METHODS

## 2.1 Study Area Description

Located between 1°50' and 2°05' East longitude and 7°00' and 7°10' North latitude, the commune of Agbangnizoun is one of the nine (09) communes in the department of Zou according to the latest territorial division. It covers an area of 244 km² and is bounded to the North by the communes of Abomey and Djidja (Zou), to the South by the commune of Lalo (Couffo), to the East by the communes of Bohicon and Zogbodomey (Zou), and to the West by the commune of Klouékanmè, Couffo department (Fig.1). It comprises 10 districts and 53 villages and urban neighborhoods. It is located 16 km from the departmental capital (Abomey), approximately 151 km from Cotonou, and spans a maximum edge-to-edge distance of 22 km in length(HOUNLELOU et al. 2022).

`\

Fig. 1: Map of some wells in the commune of Agbangnizoun

## 2.2 Study Sampling

A total of 54 samples were collected during the dry season (December 2023 to March 2024), comprising 27 samples for physico-chemical analysis and 27 samples for bacteriological analysis. These samples were taken from 27 wells numbered W1 to W27 (Table 1) to cover the entire commune and obtain an accurate overview of the physico-chemical and bacteriological quality. To obtain a representative sample from the well, collections were made using the container employed by the well owners, i.e., with the small bucket they customarily use to draw water. After drawing water from the well, sampling was performed following the method applied during borehole water sampling. For physico-chemical analysis, 1L polyethylene bottles, previously cleaned and rinsed with distilled water, were used. After drawing water from the well, the bottle was rinsed thoroughly with the sample before proceeding with collection. For bacteriological analysis, 500mL borosilicate glass bottles, previously sterilized in an autoclave at 121.1°C for 15 minutes, were used. After drawing water from the well, the sample was collected; the bottles were not completely filled to allow aerobic microbes potentially present in the water to survive until analysis. To avoid any contamination of the water by ambient air during sampling, a burner was used. Finally, once sampling was completed, the water samples were placed in a cooler containing ice pack, transported to the laboratory, and stored at 4°C. For each well, total depth was measured in situ using a graduated measuring tape. Depth was defined as the vertical distance between the upper rim of the well and the bottom (substratum or accumulated sediments).

Table 1: Samples coding

| Localities | Well Code | Localities | Well Code | Localities | Well Code |
| --- | --- | --- | --- | --- | --- |
| Wanli | W1 | N'gbénoudo | W10 | Zounme centre | W19 |
| Gbozou 2 | W2 | Dovokpè | W11 | Dodji | W20 |
| Tanta 1 | W3 | Adingnigon centre | W12 | Zoungoudo centre | W21 |
| Ahissatogon | W4 | Fondanou | W13 | Zoungoudo centre2 | W22 |
| Danli | W5 | Aïmèvo | W14 | Akodebakou centre | W23 |
| Abodéslamè | W6 | Aglali | W15 | Avali | W24 |
| Kinmin | W7 | Zounme centre | W16 | Abigo | W25 |
| Vodjè 2 | W8 | Dodomè | W17 | Tohouéta | W26 |
| Agbozoundji | W9 | kinkpèslamè | W18 | Adjahakpa /akohoun | W27 |

## 2.3 Physicochemical and Bacteriological Analyses

### 2.3.1 Physicochemical analyses

Water temperature, electrical conductivity (EC), and pH were measured in situ at each sampling point using a calibrated portable multi-parameter meter (ADWA AD8000).All remaining physicochemical parameters were determined in the laboratory following standard analytical procedures recommended by Rodier (KAHOUL and TOUHAMI 2014).Sodium (Na⁺) and potassium (K⁺) concentrations were measured by flame photometry, while calcium (Ca²⁺) and magnesium (Mg²⁺) were determined by complexometric titration with EDTA(Lal et al. 2025; Karita and Kaneta 2016).  Major anions were analyzed using the following methods: colorimetric determination for sulfates (SO₄²⁻), cadmium reduction for nitrates (NO₃⁻), argentometric titration for chlorides (Cl⁻), and acid titration with 0.01 N HCl for bicarbonates (HCO₃⁻).

### 2.3.2 Bacteriological analyses

Bacteriological analyses were conducted to assess fecal contamination of the well waters using indicator microorganisms rather than attempting a comprehensive inventory of microbial species. The selected indicators, total coliforms, Escherichia coli, and intestinal enterococci are commonly used in drinking water quality monitoring due to their association with fecal pollution and potential health risks.

Microbiological analyses were performed according to AFNOR standards using the membrane filtration method. For total viable microorganisms, the pour-plate technique was applied on Tryptone Glucose Yeast Extract Agar (TGEA). After preparation of decimal dilutions, samples were incubated under standardized conditions, and results were expressed as colony-forming units (CFU), accounting for dilution factors(Hadji Feyrouz 2020).

Total coliforms and E. coli were enumerated using the multiple-tube fermentation technique, including presumptive tests for total coliforms and confirmatory tests for E. coli (Gunawardane and Priyadarshani 2025). Results were interpreted using the MacGrady statistical tables to estimate bacterial concentrations per 100 mL of water.

Intestinal enterococci were determined through a two-step procedure involving a presumptive test on Rothe medium and a confirmatory test on Eva Litsky medium. Incubation was carried out at 37 °C for 24–48 h. Positive results were identified by turbidity and characteristic pellet formation, and concentrations were quantified using MacGrady tables(Kabore et al. 2018).

### 2.3.3 Pollution Index and heatmap analysis

Groundwater pollution was assessed using a composite Pollution Index (PI) integrating physicochemical and bacteriological parameters (pH, E. coli, total coliforms, fecal streptococci, Cl⁻, SO₄²⁻, NO₃⁻, and total hardness). All variables were standardized using z-score normalization:

Zij=Xij-μjσj

where Xijrepresents the value of parameter jin well i, and μjand σjare the corresponding mean and standard deviation.

Because pH reflects deviation from neutrality rather than monotonic pollution, it was treated separately using a centered absolute deviation:

ZpH,i=∣pHi-7.51.5∣

corresponding to the midpoint (7.5) and half-width (1.5) of the recommended range (6.5–8.5).

The Pollution Index was calculated as:

PIi=1nj=1n∣Zij∣

where n  is the number of parameters. Higher PI values indicate greater relative pollution. A heatmap of standardized scores was generated to visualize spatial contrasts and identify dominant pollution drivers among wells.

- RESULTS AND DISCUSSION

## 3.1 RESULTS

The physicochemical and microbiological characteristics of the well waters analyzed in the municipality of Agbangnizoun are presented in Table 2.

Table 2: Average Values of physico-chemical and microbiological parameters of the studied Well Waters

| Wells | CE (μS/cm) | T(°C) | pH | Ca2+ (mg/L) | Mg2+ (mg/L) | Na+ (mg/L) | K+ (mg/L) | HCO3- (mg/L) | CO32- mg/L | Cl- (mg/L) | SO42- (mg/L) | NO3- (mg/L) | E. coli (UFC/100mL) | C.T (UFC/100mL) | FS (UFC/100mL) | Total depth (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| W1 | 227 | 28 | 4.97 | 56.6 | 16 | 18.5 | 0.323 | 27 | 6 | 16.89 | 6.6 | 23 | 27 | 8 | 34.97 | 18 |
| W2 | 208 | 30 | 7.1 | 45 | 16.7 | 16.7 | 0.32 | 27 | 6.5 | 14.7 | 5.8 | 23 | 20 | 3 | 17.1 | 34 |
| W3 | 301 | 23 | 5.8 | 30 | 6.6 | 6.6 | 0.327 | 10.29 | 4 | 34.7 | 5.8 | 47.5 | 30 | 3 | 75.8 | 23 |
| W4 | 259 | 24 | 4.97 | 45 | 23.4 | 23.4 | 0.27 | 34 | 9 | 39.5 | 5.8 | 28.5 | 29 | 4 | 34.97 | 17 |
| W5 | 250 | 25 | 5.3 | 23 | 4.5 | 4.5 | 0.32 | 40 | 5 | 45.8 | 4.64 | 41.5 | 25 | 5 | 55.3 | 40 |
| W6 | 235 | 23 | 4.97 | 43.9 | 6.8 | 6.8 | 0.068 | 34 | 6 | 6.6 | 35 | 39.5 | 35 | 3 | 14.97 | 34 |
| W7 | 300 | 24 | 7.1 | 34.5 | 9.8 | 9.8 | 0.05 | 35 | 4 | 4.7 | 35.7 | 29.5 | 30 | 4 | 37.1 | 30 |
| W8 | 209 | 23 | 5.3 | 62.3 | 16 | 16 | 0.068 | 24 | 10 | 17.04 | 25 | 47.8 | 29 | 3 | 85.3 | 25 |
| W9 | 211 | 31 | 5.8 | 45.6 | 4.6 | 4.6 | 0.068 | 20 | 4 | 24.64 | 35.9 | 47.5 | 21 | 31 | 75.8 | 18 |
| W10 | 279 | 26 | 4.97 | 43.2 | 19 | 19 | 0.327 | 23 | 9 | 29.54 | 5.8 | 39.5 | 29 | 6 | 24.97 | 42 |
| W11 | 224 | 27 | 5.3 | 34.2 | 31 | 31 | 0.32 | 15 | 7 | 37.54 | 6.89 | 27.5 | 24 | 17 | 45.3 | 40 |
| W12 | 292 | 28 | 7.4 | 45.8 | 30 | 30 | 0.325 | 34 | 4 | 24.35 | 5.6 | 28.5 | 22 | 8 | 67.4 | 38 |
| W13 | 236 | 27 | 5.3 | 55 | 14.5 | 14.5 | 0.327 | 23 | 7 | 37.67 | 5.8 | 37.4 | 36 | 47 | 35.3 | 22 |
| W14 | 300 | 29 | 5.3 | 55.6 | 17.8 | 17.8 | 0.32 | 14 | 6 | 36.89 | 6.09 | 34.5 | 30 | 39 | 55.3 | 27 |
| W15 | 307 | 30 | 6.8 | 42.3 | 25.6 | 25.6 | 0.323 | 15 | 5 | 25.72 | 5.04 | 45.5 | 37 | 10 | 56.8 | 24 |
| W16 | 278 | 31 | 4.94 | 23 | 24.5 | 24.5 | 0.32 | 24 | 4.2 | 34.5 | 3.89 | 23.78 | 28 | 41 | 44.94 | 30 |
| W17 | 230 | 28 | 5.67 | 30 | 26.7 | 26.7 | 0.068 | 33 | 5.5 | 25.5 | 4.64 | 34.5 | 23 | 28 | 35.67 | 25 |
| W18 | 221 | 29 | 7.2 | 49.2 | 29 | 29 | 0.327 | 23 | 10 | 16.02 | 5.05 | 41.5 | 22 | 59 | 27.2 | 19 |
| W19 | 204 | 24 | 5.8 | 35.4 | 30.4 | 30.4 | 0.068 | 31 | 4.8 | 24.8 | 4.64 | 39.5 | 24 | 54 | 95.8 | 41 |
| W20 | 220 | 27 | 6.2 | 55 | 30 | 30 | 0.323 | 23 | 8.7 | 28.7 | 5.8 | 23.58 | 20 | 47 | 96.2 | 45 |
| W21 | 278 | 28 | 5.4 | 55.6 | 27.7 | 27.7 | 0.068 | 21 | 6.54 | 36.54 | 5.72 | 34.5 | 28 | 38 | 65.4 | 38 |
| W22 | 278 | 31 | 5.3 | 42.3 | 19 | 17.8 | 0.32 | 34 | 9 | 44.35 | 6.89 | 27.5 | 28 | 11 | 75.3 | 32 |
| W23 | 230 | 28 | 6.8 | 43.9 | 31 | 25.6 | 0.323 | 40 | 5 | 37.67 | 5.6 | 28.5 | 23 | 48 | 66.8 | 27 |
| W24 | 221 | 29 | 4.94 | 34.5 | 30 | 24.5 | 0.32 | 34 | 6 | 36.89 | 5.8 | 37.4 | 21 | 19 | 84.94 | 20 |
| W25 | 204 | 24 | 5.67 | 62.3 | 14.5 | 26.7 | 0.068 | 35 | 4 | 45.72 | 6.09 | 34.5 | 24 | 14 | 45.67 | 31 |
| W26 | 220 | 26 | 7.2 | 45.6 | 17.8 | 29 | 0.068 | 24 | 10 | 44.5 | 5.04 | 45.5 | 290 | 16 | 87.2 | 35 |
| W27 | 278 | 27 | 6.65 | 43.2 | 25.6 | 30.4 | 0.327 | 34 | 4 | 45.5 | 3.89 | 23.78 | 38 | 27 | 66.65 | 32 |

### 3.1.1 Physicochemical Characteristics of Well Water

The mean pH of the groundwater samples was 5.86 ± 0.71, ranging from 4.94 to 7.40. Among the 27 sampled wells, 19 exhibited pH values below 6.5, indicating predominantly acidic conditions. Electrical conductivity showed a mean value of 248.15 µS/cm, with values ranging from 204 to 307 µS/cm. The average water temperature was 27.04 °C, varying between 23 and 31 °C.

Cationic Composition: The concentrations of major cations showed the following mean values. Calcium (Ca²⁺) averaged 43.78 mg/L, with concentrations ranging from 23.0 to 62.3 mg/L. The highest Ca²⁺ concentrations (> 55.0 mg/L) were recorded in wells W1, W8, W14, W21, and W25. Magnesium (Mg²⁺) had a mean concentration of 20.32 mg/L, ranging from 4.5 to 31.0 mg/L. Sodium (Na⁺) concentrations averaged 21.00 mg/L (range: 4.5–31.0 mg/L). Potassium (K⁺) exhibited low concentrations, with a mean of 0.23 ± 0.09 mg/L and values ranging from 0.05 to 0.33 mg/L.

Anionic Composition: Nitrate (NO₃⁻) concentrations averaged 34.64 ± 8.92 mg/L, with values ranging from 23.0 to 47.8 mg/L. Twenty-two out of 27 wells exhibited nitrate concentrations exceeding 25 mg/L, with the highest levels observed in wells W3, W8, and W9. Chloride (Cl⁻) concentrations showed a mean of 30.26 mg/L, ranging from 4.7 to 45.8 mg/L. Bicarbonate (HCO₃⁻) concentrations averaged 27.08 mg/L, with values between 10.29 and 40 mg/L. Sulfate (SO₄²⁻) concentrations remained relatively low, with a mean of 9.57 mg/L and a range of 3.89 to 35.90 mg/L. Carbonate (CO₃²⁻) concentrations ranged from 4 to 10 mg/L, with a mean value of 6.31 mg/L.

Total Hardness (Fig. 2): Total hardness, expressed as CaCO₃ equivalent, ranged from 75.95 to 260.5 mg/L, with an average value of 192.74 mg/L. The highest hardness values (> 235.0 mg/L) were recorded in wells W18, W20, W21, and W23, whereas wells W3 and W5 exhibited the lowest hardness levels, although still exceeding 75.0 mg/L.

Characteristic ionic ratios (Fig. 2): The mean value of the (Na⁺ + K⁺)/Cl⁻ ratio was 0.83, with values ranging from 0.11 to 2.10. The NO₃⁻/Cl⁻ ratio showed a mean of 1.58, with 55.56% of the wells exhibiting ratios greater than 1. The SO₄²⁻/Cl⁻ ratio displayed a mean value of 0.74, while the average Ca²⁺/Mg²⁺ ratio was 2.86.

Hydrochemical Classification (Fig.3): The Piper diagram (Figure 2) indicates that the well waters are mainly distributed between two hydrochemical facies: sodium–potassium chloride or sodium sulfate facies, and calcium–magnesium chloride or calcium–magnesium sulfate facies. Most samples plot within the central field of the diagram, reflecting mixed cationic composition without a dominant single ion. A noticeable dispersion is observed along the cationic axis, with some samples tending toward the calcium (Ca²⁺) pole, while others shift toward the sodium (Na⁺) pole. On the anionic side, most samples cluster near the bicarbonate (HCO₃⁻) field. A distinct subgroup of samples, corresponding to wells with the highest chloride and nitrate concentrations, deviates toward the anionic sector dominated by chloride (Cl⁻) and nitrate (NO₃⁻).

Fig. 2: Characteristic ionic ratios (a) and Total hardness, expressed as CaCO₃ equivalent (b)

Fig. 3: Classification of well water using Piper diagram

### 3.1.2 Results of Bacteriological Analyses

The microbiological analyses of well water samples, revealed a systematic presence of fecal contamination indicators in all sampled wells.

Escherichia coli (EC) was detected in all 27 samples, with concentrations ranging from 20.0 to 290.0 CFU/100 mL (mean: 36.78 ± 51.42 CFU/100 mL). The highest contamination level was observed in well W26 (290.0 CFU/100 mL). Total coliforms were also identified in every well, with concentrations varying between 3.0 and 59.0 CFU/100 mL (mean: 21.96 ± 16.03 CFU/100 mL; median: 17.0 CFU/100 mL). Fecal streptococci (FS) were detected in all samples, with concentrations ranging from 14.97 to 96.20 CFU/100 mL (mean: 55.86 ± 23.45 CFU/100 mL).

The fecal streptococci to Escherichia coli ratio (FS/EC) showed a mean value of 2.07 ± 1.15, with values ranging from 0.30 to 4.81 (Fig. 4). The distribution of this ratio indicates that 55.6% of the samples (15/27) exhibited SF/EC values greater than 2.0.

Fig. 4: Ratio Fecal streptococci to Escherichia coli (FS/EC)

### 3.1.3 Results of Pollution Index and heatmap analysis

The Pollution Index (PI), calculated as the mean of the absolute values of standardized scores, highlights a pronounced spatial heterogeneity in groundwater pollution across the study area (Fig.5). The highest PI values were recorded for wells W6 to W9, as well as W26, indicating a more intense overall level of contamination at these sites.

Cross-analysis with the heatmap reveals that elevated PI values in wells W6–W9 are primarily driven by high sulfate and nitrate concentrations, suggesting a dominant influence of physicochemical contamination. In contrast, well W26 is mainly characterized by marked bacteriological contamination, indicating a different pollution regime dominated by microbiological inputs.

Conversely, wells W1, W2, W12, W16, W17, W22, and W27 exhibit comparatively lower PI values, reflecting a generally better overall water quality relative to the other sampling points.

Fig. 5. Heatmap of relative pollution levels across wells. Colors indicate relative pollution intensity, ranging from low pollution (violet–blue) to high pollution (green–yellow–red) (a) and Pollution Index (b).

## 3.2 DISCUSSION

### 3.2.1 Physicochemical quality of well water

The physicochemical results highlight several constraints that may affect the suitability of well water for human consumption and domestic uses in the municipality of Agbangnizoun. pH values indicate a predominance of acidic conditions, with the majority of samples exhibiting pH values below the range recommended by Beninese standards and World Health Organization (WHO) guidelines (6.5–8.5)(Cotruvo 2017).These findings are consistent with those reported in other regions of Benin and West Africa, where weakly buffered groundwater has been observed in shallow aquifers developed within weathered basement formations(Dansou and Odoulami 2015; Miriac et al. 2020). Acidic pH conditions may promote corrosion of hydraulic structures and influence the mobility of certain chemical elements in water(Wasim et al. 2018).

Electrical conductivity values reflect low to moderate mineralization, characteristic of aquifer systems fed by recent recharge and marked by limited water–rock interactions. Comparable conductivity ranges have been reported in similar hydrogeological settings across West Africa (Talabi et al. 2013), suggesting regional consistency in mineralization processes.

Nitrate concentrations represent one of the most sensitive chemical parameters observed in this study. Although measured values remain below the WHO guideline value of 50 mg/L(Cotruvo 2017; Sadler et al. 2016), a high proportion of wells exhibit nitrate concentrations exceeding 25 mg/L, a threshold frequently used as an indicator of anthropogenic influence in groundwater(Cámaro García and Dwyer 2021). This pattern suggests a significant contribution from agricultural and domestic activities, particularly the use of nitrogen-based fertilizers and the infiltration of wastewater, as reported in numerous rural areas of sub-Saharan Africa (Ebong et al. 2025). Such contamination may pose potential health risks, especially for vulnerable populations(Chébékoué 2008).

Other major anions (chlorides, sulfates, and carbonates) display relatively low concentrations and generally remain below the limit values established by national and international standards(Gbohaida et al. 2016; Cotruvo 2017). Nevertheless, their widespread presence across most wells reflects the combined influence of natural inputs and low-intensity anthropogenic contributions.

Cationic composition is dominated by calcium and magnesium, resulting in total hardness values ranging from moderately hard to hard water. Although water hardness does not constitute a direct health risk, it may cause domestic inconveniences such as scaling of equipment and increased detergent consumption(Mons et al. 2007). The Ca²⁺/Mg²⁺ ratio exceeding unity in most samples suggests a dominant contribution from calcium-bearing minerals, consistent with weathering processes affecting local geological formations(Promilton et al. 2025).

Ionic ratio analysis provides additional insights into the hydrochemical mechanisms governing groundwater composition. The (Na⁺ + K⁺)/Cl⁻ ratio, which is on average below unity, indicates a limited contribution from marine-derived salinity and highlights the predominance of continental processes(Lara et al. 2026). Elevated NO₃⁻/Cl⁻ ratios in several wells emphasize the relative importance of nitrate sources compared to chlorides. SO₄²⁻/Cl⁻ ratios, generally below unity, suggest a minor relative contribution of sulfates to overall mineralization, although a few wells exhibit distinct signatures that may reflect localized hydrochemical conditions.

Hydrochemical classification based on the Piper diagram reveals a predominance of calcium–magnesium bicarbonate facies, with mixed facies observed in some wells. This distribution indicates moderate hydrochemical evolution of groundwater, influenced by both natural rock weathering processes and localized anthropogenic inputs.

### 3.2.2 Bacteriological quality of well water

Bacteriological results reveal widespread fecal contamination of the studied well waters. The systematic detection of Escherichia coli, total coliforms, and fecal streptococci in all samples indicates that these waters do not meet the microbiological criteria for drinking water established by the WHO, which require the complete absence of fecal contamination indicators in 100 mL of water intended for consumption(Khan 2020; Salamandane et al. 2021).

This generalized contamination reflects the vulnerability of shallow groundwater to fecal inputs, which has also been documented in comparable West African settings(Orji et al. 2020) Several factors may contribute, including insufficient physical protection of wells, frequent proximity of latrines and domestic pollution sources (often within 10 m), and water-drawing practices that promote secondary contamination. Local hydroclimatic conditions, particularly intense rainfall events, may further facilitate the rapid transfer of microorganisms to the groundwater through infiltration.

Analysis of the fecal streptococci to Escherichia coli ratio (FS/EC) provides additional information on the nature and persistence of contamination. High ratios observed in a significant proportion of wells are generally associated with older contamination or a predominance of animal-derived sources, given the greater environmental persistence of fecal streptococci(Houédakor et al. 2025). However, the coexistence of different ratio values across the study area suggests multiple contamination sources and diverse contamination dynamics.

### 3.2.3 Multi-parameter integration: contribution of the Pollution Index and heatmap

Beyond individual parameter analysis, integration of physicochemical and bacteriological results using the Pollution Index (PI) reveals pronounced spatial heterogeneity in groundwater quality across the study area. Elevated PI values observed in wells W6 to W9 and W26 indicate more severe overall degradation, confirming that the pressures identified are not uniformly expressed at the local scale.

Cross-analysis with the heatmap clarifies the relative contributions of individual parameters to overall pollution. In wells W6–W9, high PI values are primarily driven by elevated sulfate and nitrate concentrations, suggesting the dominance of anthropogenic dissolved inputs, in agreement with trends identified in the chemical analysis. In contrast, well W26 is characterized by a major contribution from bacteriological parameters, indicating heightened vulnerability to fecal contamination rather than mineralization processes.

Wells exhibiting lower PI values (W1, W2, W12, W16, W17, W22, and W27) display more balanced profiles, characterized by moderate scores across all considered parameters. This differentiation highlights the value of the integrated approach, which not only enables ranking of wells according to their relative pollution levels but also facilitates identification of the dominant factors responsible for water quality degradation.

In this context, the PI constitutes a relevant tool for synthesizing and spatially comparing groundwater quality, complementing conventional analyses and providing methodological support for prioritizing management and protection actions targeting the most vulnerable water points.

- CONCLUSION

This study provides a comprehensive assessment of the physicochemical and bacteriological quality of well water used by populations in the commune of Agbangnizoun. The results indicate that, although most major inorganic parameters remain within national and international guideline values, several constraints compromise the suitability of these waters for direct human consumption. These include a widespread acidic pH, elevated nitrate concentrations in a significant proportion of wells, and moderate to high water hardness reflecting the influence of local geological formations.

The bacteriological analyses reveal a generalized fecal contamination of all sampled wells, evidenced by the systematic detection of Escherichia coli, total coliforms, and fecal streptococci. This widespread microbial pollution highlights the high vulnerability of shallow groundwater to surface-derived contamination and renders the studied water sources unsuitable for drinking without prior treatment. The SF/EC ratios further suggest the coexistence of persistent and possibly older contamination sources, alongside more recent fecal inputs depending on site-specific conditions.

The Pollution Index (PI), derived from standardized physicochemical and microbiological parameters, reveals marked spatial heterogeneity in groundwater quality across the study area. Wells W6–W9 and W26 exhibit the highest pollution levels, driven respectively by elevated sulfate–nitrate concentrations and pronounced bacteriological contamination, while several other wells show comparatively lower levels of overall pollution. This integrative approach proved effective in highlighting priority sites for intervention and in facilitating comparative analysis between wells.

It should be noted that the present study represents a snapshot of groundwater quality under specific hydrological conditions and does not account for seasonal variability, which may significantly influence both chemical composition and microbial dynamics. Future investigations should therefore incorporate multi-seasonal monitoring and a more detailed characterization of pollution sources. Nevertheless, the findings underscore the urgent need for improved protection of wells, enhanced sanitation practices, and the promotion of household-level water treatment to reduce health risks and ensure safer access to drinking water in Agbangnizoun.

Disclaimer (Artificial intelligence)

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

REFERENCES

- Brender, Jean D. 2020. “Human Health Effects of Exposure to Nitrate, Nitrite, and Nitrogen Dioxide.” In Just Enough Nitrogen: Perspectives on How to Get There for Regions with Too Much and Too Little Nitrogen. Springer.

- Cámaro García, Walther C. A., and Ned Dwyer. 2021. The Status of Ireland’s Climate, 2020. Online version. With Environmental Protection Agency. EPA Research Report, no. 386. Environmental Protection Agency.

- Chaudhary, Indra Jeet, Ratna Chauhan, Sanjay Sarjerao Kale, et al. 2025. “Groundwater Nitrate Contamination and Its Effect on Human Health: A Review.” Water Conservation Science and Engineering 10 (1) : 33.

- Chébékoué, Sandrine Fleur. 2008. Évaluation Du Risque Cancérigène Associé à La Contamination de l’eau Potable de Puits Municipaux Par Les Nitrates/Nitrites Dans Certaines Régions Rurales Du Québec. [Assessment of carcinogenic risk associated with nitrate/nitrite contamination of municipal well water in selected rural regions of Quebec]

- Cotruvo, Joseph A. 2017. “2017 WHO Guidelines for Drinking Water Quality: First Addendum to the Fourth Edition.” Journal‐American Water Works Association 109 (7): 44–51.

- Dansou, Brice S., and Léocadie Odoulami. 2015. “Facteurs de Dégradation Des Eaux de Puits à Usage Domestique Dans La Commune de Pobè Au Sud-Est Du Bénin.” [Factors Contributing to the Degradation of Well Water for Domestic Use in the Commune of Pobè, Southeastern Benin]. Afrique Science 11: 367–76.

- Dovonou, Flavien Edia, Abdoukarim Alassane, Vidédji Naéssé Adjahossou, Bibiane Agbodo, Roufai Djibril, and Daouda Mama. 2017. “Impacts de l’assainissement Autonome Sur La Qualité Des Eaux de Puits Dans La Commune de Sèmè-Podji (Sud-Bénin).” [Impacts of On-Site Sanitation on Well Water Quality in the Commune of Sèmè-Podji (Southern Benin)]. International Journal of Biological and Chemical Sciences 11 (6): 3086–99.

- Ebong, Ebong D., Ukam O. Ukam, Oualid Melouah, Glory ME Nja, and Nguyen K. Dung. 2025. “Groundwater Quality Evaluation and Public Health Implications of Its Use in Obudu, Southeastern Nigeria.” Environmental Science and Pollution Research 32 (56): 30748–65.

- Egbueri, Johnbosco C., Johnson C. Agbasi, Leonard N. Onuba, Nchekwube D. Nweke, Henry C. Uwajingba, and Sani I. Abba. 2025. “Groundwater Development Within the Nigerian Crystalline and Sedimentary Aquifers: Challenges and Opportunities.” Groundwater in Developing Countries: Case Studies from MENA, Asia and West Africa, 297–325.

- Elisante, Eliapenda, and Alfred NN Muzuka. 2017. “Occurrence of Nitrate in Tanzanian Groundwater Aquifers: A Review.” Applied Water Science 7 (1): 71–87.

- Gbohaida, Virginie, DC Pascal Agbangnan, Mafuyu Bucku Ngossanga, et al. 2016. “Etude de La Qualité Physico-Chimique de l’eau de Boisson Dans Deux Localités Du Bénin : Cotonou et Dassa-Zoumè.” [Study of the Physico-Chemical Quality of Drinking Water in Two Localities in Benin : Cotonou and Dassa-Zoumè]. International Journal of Biological and Chemical Sciences 10 (1): 422–34.

- Gunawardane, MM, and KPR Priyadarshani. 2025. “Decoding the Multiple Tube Fermentation Method Used in Coliform Testing.” Journal of Multidisciplinary & Translational Research 10 (1).

- Hadji Feyrouz, Boucceredj Imane. 2020. Analyse Physico-Chimique et Microbiologique de l’eau. [Physico-Chemical and Microbiological Analysis of Water]

- Houédakor, Koko Zébéto, Djiwonou Koffi Adjalo, Benoît Danvide, Henri Sourou Totin Vodounon, and Ernest Amoussou. 2025. “Groundwater Characteristics and Quality in the Coastal Zone of Lomé, Togo.” Water 17 (12): 1813.

- HOUNLELOU, Franck, Léopold DEGBEGNON, and Mama DJAOUGA. 2022. SIG et Gestion Du Registre Foncier Urbain à l’ère de La Décentralisation Dans l’arrondissement d’Agbangnizoun. [GIS and Urban Land Registry Management in the Era of Decentralization in the Agbangnizoun District]

- Hounsounou, Espérance Olive, M. Agassounon Djikpo Tchibozo, L. Ayi-Fanou, et al. 2017. “Indicateurs de Pollution Bactériologique Des Eaux de Puits à Usages Domestiques et Risques Sanitaires Dans Le Sixième Arrondissement de Cotonou Au Sud-Bénin.” [Bacteriological Pollution Indicators of Well Water for Domestic Use and Health Risks in the Sixth Arrondissement of Cotonou, Southern Benin]. Afrique Science 13: 425–34.

- Kabore, Aminata, Boubacar Savadogo, Jacques Savadogo, et al. 2018. “Problématique de La Qualité Microbiologique de l’eau de Boisson En Milieu Scolaire Dans Les Zones Rurales Au Burkina Faso.” [Microbiological Quality Issues of Drinking Water in Schools in Rural Areas of Burkina Faso]. Sciences de La Vie, de La Terre et Agronomie 5 (2).

- KAHOUL, Mohamed, and Mohamed TOUHAMI. 2014. “Évaluation de La Qualité Physico-Chimique Des Eaux de Consommation de La Ville d’Annaba (Algérie).” [Assessment of the Physico-Chemical Quality of Drinking Water in the City of Annaba (Algeria)]. LARHYSS Journal P-ISSN 1112-3680/E-ISSN 2521-9782, no. 19.

- Karita, Shingo, and Takashi Kaneta. 2016. “Chelate Titrations of Ca2+ and Mg2+ Using Microfluidic Paper-Based Analytical Devices.” Analytica Chimica Acta 924: 60–67.

- Khan, Farhan Mohammad. 2020. Escherichia Coli (E. Coli) as an Indicator of Fecal Contamination in Water: A Review.

- Lal, Bharat, Satendra Singh Sengar, Harish Kumar, et al. 2025. “Impact of Poor-Quality Irrigation Water on Soil Systems: A Hydrogeochemical Study from Central India.” Earth Systems and Environment, 1–25.

- Lapworth, DJ, DCW Nkhuwa, J. Okotto-Okotto, et al. 2017. “Urban Groundwater Quality in Sub-Saharan Africa: Current Status and Implications for Water Security and Public Health.” Hydrogeology Journal 25 (4): 1093–116.

- Lara, Zoraya Y. Martínez, Mónica C. Cantero Benitez, Adriana S. Raciny Soto, Neider A. Burgos Ortiz, and Ángela I. Villalba López. 2026. “Origin and Hydrogeochemical Processes in the Salinization of a Coastal Aquifer in the Colombian Caribbean1.” Brazilian Journal of Agricultural and Environmental Engineering 30 (1): e291614.

- Liddle, Elisabeth S., Sarah M. Mager, and Etienne L. Nel. 2015. “The Suitability of Shallow Hand Dug Wells for Safe Water Provision in Sub-Saharan Africa: Lessons from Ndola, Zambia.” Applied Geography 57: 80–90.

- Marc, Sohounou, Vissin Expédit Wilfrid, Sintondji Luc Olivier, Agbossou K. Euloge, and Edorh A. Patrick. 2017. “Physico-Chemical and Microbiological Qualities of Water from Wells, Drillings and Tanks Used as Drinking Water in the Municipality of Allada (Benin, West Africa).” European Scientific Journal 13: 15–69.

- Miriac, Ahouansè Sènan Dimitri, A. Noukpo, and HS Christophe. 2020. “Eau de Consommation et Maladies Hydriques Dans La Commune de Lokossa Au Sud-Ouest de La République Du Bénin (Afrique de l’Ouest).” [Drinking Water and Waterborne Diseases in the Commune of Lokossa, Southwestern Republic of Benin (West Africa)]. European Scientific Journal, ESJ 16 (15): 393–4017.

- Mons, M., H. Van Dijk, D. Gatel, et al. 2007. “Drinking Water Hardness : Reasons and Criteria for Softening and Conditioning of Drinking Water.” Global Water Research Coalition: London, UK.

- Oon, Yoong-Ling, Yoong-Sin Oon, Muhammad Ayaz, Min Deng, Lu Li, and Kang Song. 2023. “Waterborne Pathogens Detection Technologies: Advances, Challenges, and Future Perspectives.” Frontiers in Microbiology 14: 1286923.

- Orji, MU, SC Onuorah, MO Ifemeje, and OR Umeh. 2020. “Effects of Depth and Seasons on the Physicochemical and Bacteriological Quality of Selected Well Water Samples in Awka Urban, Anambra State, Nigeria.” Asian Journal of Biology 10 (1): 1–28.

- Promilton, A. Antony Alosanai, A. Antony Ravindran, V. Stephen Pitchaimani, J. Vinoth Kingston, and Shankar Karuppannan. 2025. “Comprehensive Hydrogeochemical Characterization and Seasonal Water Quality Index Analysis for Sustainable Groundwater Management in Valliyur Region, Southern Tamil Nadu, India.” Scientific Reports 15 (1): 33251.

- Sadler, Ross, Brooke Maetam, Benjamin Edokpolo, et al. 2016. “Health Risk Assessment for Exposure to Nitrate in Drinking Water from Village Wells in Semarang, Indonesia.” Environmental Pollution 216: 738–45.

- Salamandane, Acácio, Filipa Vila-Boa, Manuel Malfeito-Ferreira, and Luísa Brito. 2021. “High Fecal Contamination and High Levels of Antibiotic-Resistant Enterobacteriaceae in Water Consumed in the City of Maputo, Mozambique.” Biology 10 (6): 558.

- Talabi, AO, OL Afolagboye, MN Tijani, JA Aladejana, and AK Ogundana. 2013. “Hydrogeochemical Assessment of Surface Water in the Central Part of Ekiti-State, Southwestern Nigeria.” American Journal of Water Resources 1 (4): 56–65.

- Tossou, Yao Yélidji Joël, Philippe Orban, Julie Gesels, et al. 2017. “Hydrogeochemical Mechanisms Governing the Mineralization and Elevated Fluoride F-Contents in Precambrian Crystalline Aquifer Groundwater in Central Benin, Western Africa.” Environmental Earth Sciences 76 (20): 691.

- VODOUNOU, Jean Bosco Kpatindé, and Alfred Okoungla DENONSI. 2018. “Caractéristiques Physico-Chimiques et Bactériologiques Des Eaux de Puits Dans La Ville de Parakou, Centre-Bénin.” Afrique SCIENCE 14 (3): 336–52.

- Wamsley, Miriam. 2014. Evidence of Ground Water Contamination by On-Site Wastewater Systems.

- Ward, Jade Suzanne. 2020. Investigating Novel Approaches for Detecting Microbial Contamination in Drinking Water in Africa.

- Wasim, Muhammad, Shahrukh Shoaib, NM Mubarak, Inamuddin, and Abdullah M. Asiri. 2018. “Factors Influencing Corrosion of Metal Pipes in Soils.” Environmental Chemistry Letters 16 (3): 861–79.

- Yuan, Ruiqiang, Zhibin Li, and Siyu Guo. 2023. “Health Risks of Shallow Groundwater in the Five Basins of Shanxi, China: Geographical, Geological and Human Activity Roles.” Environmental Pollution 316: 120524.

- Zhang, Minmin, Lishu Wang, Qiaojing Zhao, Jinxi Wang, and Yuzhuang Sun. 2024. “Hydrogeochemical and Anthropogenic Controls on Quality and Quantitative Source-Specific Risks of Groundwater in a Resource-Based Area with Intensive Industrial and Agricultural Activities.” Journal of Cleaner Production 440: 140911.