Water Quality Assessment of Begum Talab Lake in Vijayapura District for Drinking Purpose Using WQI (Water Quality Index)

ABSTRACT

The present study evaluated the drinking water quality of Begum Talab Lake, located in Vijayapura district, Karnataka, India, using the Water Quality Index (WQI) approach. Surface water samples were collected monthly from July 2024 to June 2025, and major physicochemical parameters were analysed following standard APHA (1998) methods. The pH ranged from 7.70 to 8.40, remaining within the desirable limits prescribed by BIS. Dissolved oxygen varied between 4.4 and 6.2 mg/L, while biological oxygen demand and chemical oxygen demand ranged from 1.69–5.05 mg/L and 11–24 mg/L, respectively. Total alkalinity (70–120 mg/L) and hardness (75–125 mg/L) indicated moderately hard water. Total dissolved solids and total suspended solids ranged from 195–320 mg/L and 22–64 mg/L, respectively, remaining within permissible limits. Nitrate concentrations were low (0.05–0.60 mg/L), suggesting minimal nutrient pollution. The computed overall WQI value was 92.16, categorizing the lake water as “Very Good” according to CCME classification. The findings indicate that Begum Talab Lake water is largely protected and suitable for drinking purposes with minimal treatment. However, continuous monitoring and proper management practices are recommended to maintain long-term water quality sustainability.

Keywords: Freshwater, Water Quality Index, Begum Talab Lake, Dissolved oxygen

INTRODUCTION

Freshwater is essential for the sustenance of life on Earth and is primarily found in surface water bodies such as lakes, rivers, ponds, and canals. A lake is defined as a large depression in the Earth that contains slowly moving or standing water (Krishne Gowda, 2007). Although lakes occupy only a small fraction of the Earth’s surface, freshwater lakes are invaluable national resources, providing water for both irrigation and domestic consumption. Surface water constitutes a critical component of the natural environment, and its quality is highly sensitive, making it a significant concern in many countries. With growing recognition of the importance of drinking water quality for public health, as well as the impact of raw water quality on aquatic ecosystems, systematic assessment and monitoring of surface water quality have become increasingly necessary (Ouyang, 2003).

The term “water quality” encompasses not only the chemical and physical characteristics of the water column but also the condition of the physical channel necessary to sustain aquatic life. The federal Clean Water Act underscores this concept by aiming “to protect and maintain the chemical, physical, and biological integrity of the nation’s waters,” highlighting the importance of assessing both water quality and the habitats required for supporting aquatic organisms. However, water, a vital resource bestowed by nature, is increasingly subjected to pollution due to rapid urbanization and anthropogenic pressures (Bharati et al., 2016).

The use of Water Quality Indices (WQI) provides a straightforward approach to overcome many of the challenges associated with interpreting complex water quality data, enabling both the public and decision-makers to access concise and meaningful information regarding water quality (AOAC, 1995). The WQI has become a principal tool in pollution profiling studies, offering a simplified yet comprehensive assessment of water quality. Horton (1965) introduced the first formal WQI, which laid the foundation for subsequent applications in water resource management (Ott, 1978). WQI is widely regarded as one of the most effective methods for communicating water quality status to non-specialists (Pradhan et al., 2001). Numerous national-level studies have employed calculated WQI values to evaluate the water quality of various natural water resources, demonstrating its utility in environmental monitoring and management (Rajmohan, 2003).

The quality of water in lakes, reservoirs, and rivers is primarily determined by physico-chemical and biological parameters. The Water Quality Index (WQI) provides an integrated measure of overall water quality at a given location and time by combining multiple water quality parameters into a single value. Monitoring water quality is a critical tool for assessing pollution levels, evaluating the effectiveness of management practices, and guiding sustainable water resource management (Singh and Jayakumar, 2016). Consequently, systematic monitoring of aquatic resources is essential to ensure their long-term sustainability. WQIs are generally classified into two categories: physico-chemical indices and biological indices. Physico-chemical indices are derived from chemical and physical characteristics of water samples, whereas biological indices are based on measurements of biological parameters. In the present study, the WQI was calculated using hydrochemical parameters. The primary objective of this investigation was to evaluate the physico-chemical characteristics of Begum Talab Lake and assess its suitability for human consumption based on the computed WQI.

MATERIALS AND METHODS

Study area

Begum Talab Lake is located in Vijayapura district, Karnataka, India (16.794631° N latitude and 75.707293° E longitude) (Fig. 1). Built in 1651 by Mohammad Ali Shah and named after Adil Shah’s queen Jahan Begum, this reservoir was supervised by commander Afzal Khan. It played a vital role in supplying water to Vijayapura's population. Surrounded by lush greenery and lively activities along its perimeter, the site is both scenic and culturally vibrant. Along with other Bijapur landmarks, it is on UNESCO’s tentative list under Monuments and Forts of the Deccan Sultanate.

Figure 1: Map of study area (Begum Talab Lake)

Surface water samples were collected monthly from July 2024 to June 2025 at a fixed sampling location in Begum Talab Lake (Fig. 1) to assess temporal variations in physicochemical parameters. At each sampling event, three replicate samples were collected and analysed separately, and the mean values were used for statistical analysis and WQI calculation. Sampling was conducted between 8:00 AM and 10:00 AM to minimize diurnal variations in dissolved oxygen and other parameters. Physicochemical parameters, including pH, DO, BOD, COD, alkalinity, hardness, TDS, TSS, and nitrate, were analysed. pH was measured on site, while biochemical oxygen demand was determined using the Winkler method in separate 300 mL bottles. Samples were transported to the laboratory in dark ice boxes to prevent exposure to sunlight. Analyses of physicochemical variables followed the Standard Methods (APHA, 1998). The Water Quality Index (WQI) was calculated using the drinking water quality standards recommended by the World Health Organization.

Calculation of WQI

WQI=∑Qnwn∑wn

Where:

- Qn = Water quality rating

Qn=100×Vn-ViSn-Vi

- Vn = Actual value of the parameter in the water sample

- Vi = Ideal value ((0 for all parameters except pH and DO which is 7.0 and 14.6 mg/l,       respectively)

- Sn = Standard value

- wn = Unit weight of the parameter

wn=KSn

- K = Constant, calculated as

K=11S1+1S2+…1Sn

Classification of WQI according to CCME (2005).

RESULTS AND DISCUSSION

Physico-chemical analysis of water quality

Several studies have assessed lake water quality using physico-chemical parameters (Giriyappanavar et al., 2013).The physicochemical analysis of Begum Talab Lake revealed moderate seasonal variations across the study period. Most parameters remained within permissible limits of drinking water standards. Slight fluctuations were observed during monsoon and summer months, which may be attributed to runoff, evaporation, and organic load variations. The detailed interpretation of individual parameters is discussed below.  The detailed monthly variation in physicochemical parameters of Begum Talab Lake during the study period (July 2024–June 2025) is presented in Table 1, showing seasonal trends and variability in water quality.

Table 1: Monthly Variation of Physico-Chemical Parameters of Begum Talab Lake (July 2024–June 2025)

| Month | pH | DO (mg/L) | BOD (mg/L) | COD (mg/L) | Alkalinity (mg/L) | Hardness (mg/L) | TDS (mg/L) | TSS (mg/L) | Nitrate (mg/L) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| July | 7.70 | 5.10 | 3.90 | 18.00 | 100 | 105 | 230 | 40 | 0.20 |
| August | 7.75 | 5.20 | 4.00 | 19.00 | 95 | 100 | 220 | 38 | 0.18 |
| September | 8.40 | 5.30 | 4.30 | 21.00 | 110 | 120 | 250 | 45 | 0.30 |
| October | 7.85 | 5.40 | 4.50 | 20.00 | 105 | 115 | 240 | 42 | 0.25 |
| November | 7.80 | 5.60 | 3.80 | 11.00 | 85 | 90 | 210 | 30 | 0.12 |
| December | 7.90 | 5.80 | 5.05 | 17.50 | 80 | 85 | 205 | 28 | 0.10 |
| January | 7.82 | 6.00 | 3.40 | 16.00 | 75 | 80 | 200 | 25 | 0.08 |
| February | 7.95 | 6.20 | 2.80 | 15.50 | 70 | 75 | 195 | 22 | 0.05 |
| March | 7.88 | 5.50 | 3.20 | 18.50 | 90 | 95 | 220 | 35 | 0.15 |
| April | 7.92 | 5.20 | 3.60 | 19.00 | 110 | 110 | 260 | 48 | 0.22 |
| May | 7.78 | 4.60 | 1.69 | 22.50 | 120 | 125 | 300 | 60 | 0.40 |
| June | 7.82 | 4.40 | 4.80 | 24.00 | 118 | 122 | 320 | 64 | 0.60 |

pH

The overall quality of water is largely defined by its chemical characteristics. pH, for instance, is a key indicator of water suitability for various uses. At Begum Talab Lake, the average pH was 7.88 ± 0.18, remaining alkaline throughout the study period, with a maximum of 8.4 in September. These values fall within the desirable limits prescribed by BIS. Comparable findings were reported by Kodom et al. (2018) in a tropical lake in Ghana and by Singh et al. (2020) in Fateh Sagar Lake.

Dissolved oxygen (DO)

Dissolved oxygen (DO) in water bodies is shaped by factors such as temperature, photosynthesis, wind, respiration, and pollution. Its levels vary daily, seasonally, and with temperature changes (Wavde and Arjun, 2010). Dissolved oxygen (DO) levels ranged between 4.4 and 6.2 mg/l, peaking in February (summer) and reaching a minimum in June. The present findings align with those of Kodom et al. (2018) on Volta Lake, Ghana, but contrast with Kangabam et al. (2017) on Loktak Lake, Manipur, which may be attributed to comparable climatic conditions and nutrient dynamics in tropical freshwater systems.

Biological Oxygen Demand (BOD) and Chemical Oxygen Demand (COD)

The BOD test assesses the presence of organic matter in water by measuring the oxygen required by aquatic microbes to decompose biodegradable material (Sharma et al., 2013). Biological Oxygen Demand (BOD), which reflects the extent of organic matter pollution (Kumar and Bahadur, 2009), varied from 1.69 mg/l in May to 5.05 mg/l in December. Chemical oxygen demand (COD) measures the total amount of organic matter in water, both biodegradable and non-biodegradable (Khan et al., 2014). Chemical Oxygen Demand (COD) values ranged between 11.36 and 24.12 mg/l, with the lowest value recorded in November and the highest in June. Since COD provides faster results than BOD, it is generally preferred (Sharma et al., 2013).

Total alkalinity

Total alkalinity reflects the water’s buffering capacity, helping to stabilize pH (Lodh et al., 2014). It measures the ability to neutralize acids and is generally not considered a pollutant (Sharma and Kumar, 2017). The average alkalinity of Begum Talab Lake ranged between 70.0 and 120 mg/l, consistent with observations from Almatti Reservoir, Karnataka (Hulyal and Kaliwal, 2011). The high alkalinity observed in summer resulted from increased decomposition, which released CO₂ and added carbonate and bicarbonate ions, thereby raising alkalinity levels (Verma et al., 2012).

Water hardness

Water hardness primarily results from calcium and magnesium cations. Water hardness varied from 75 to 125 mg/l, with an average of 115 mg/l, classifying the lake water as moderately hard according to Sawyer (1960). Usman et al. (2018) observed higher water hardness in Mumbai’s urban lakes during summer and lower levels in winter, while Manzoor et al. (2017) and Tyor & Chawla (2012) reported much lower hardness values in Sukhna Lake.

Total dissolved solids (TDS) and total suspended solids (TSS)

According to WHO (2011) guidelines, drinking water is considered palatable when total dissolved solids (TDS) are below 600 mg/l, while concentrations above 1000 mg/l are deemed unappealing. Total dissolved solids (TDS) and total suspended solids (TSS) were recorded between 195–320 mg/l and 22–64 mg/l, respectively. For drinking water, BIS 10500 (2012) prescribes a desirable limit of 500 mg/l for TDS, extendable up to 2000 mg/l in the absence of alternative sources (Dubey et al., 2014). Thus, the observed TDS and TSS values fall within acceptable limits.

Nitrate

Inorganic nitrogen is the most common form in natural waters and serves as a key nutrient driving the growth of hydrophytes and aquatic algae (Lodh et al., 2014). Nitrate concentrations ranged from 0.05 to 0.6 mg/l, with an average of 0.25 mg/l. Similar observations were noted by Verma et al. (2012). During the monsoon, nitrogen-rich floodwaters from the catchment introduced contaminants such as sewage, elevating nitrate levels consistent with the present study. In contrast, Bhat and Pandit (2014) reported lower nitrate concentrations in Wular Lake, Kashmir during summer. Decomposing plant matter and agricultural runoff elevate nitrogen levels in surface waters. Nitrate, a key indicator of organic pollution, originates from fertilizers, rainfall, crop residues, decaying organic waste, and septic systems (Khan et al., 2014).

Water quality assessment of Begum Talab lake

Table 2: WQI classification by CCME (2005).

| WQI | Quality | Description |
| --- | --- | --- |
| 95–100 | Excellent | Water quality is protected with virtual absence of threat or impairment; conditions very close to natural or desirable levels. |
| 89–94 | Very good | Water quality is protected with a slight presence of threat or impairment; conditions close to natural or pristine levels. |
| 80–88 | Good | Water quality is protected with only minor degree of threat or impairment; conditions depart from natural or desirable levels. |
| 60–79 | Fair | Water quality is usually protected but occasionally threatened or impaired; conditions sometimes depart from natural or desirable levels. |
| 45–59 | Poor (Marginal) | Water quality is frequently threatened or impaired; conditions sometimes often depart from natural or desirable levels. |
| 0–44 | Very poor | Water quality is almost always threatened or impaired; conditions sometimes usually depart from natural or desirable levels (Unsuitable for drinking). |

Table 3: WQI of Begum Talab lake water (all values except pH are in mg/l).

| Parameter | Actual Average Value | Standard Value (Sn) | Recommended Agency | 1/Sn | Wn | Qn | QnWn |
| --- | --- | --- | --- | --- | --- | --- | --- |
| pH | 7.82 | 8.5 | ICMR / BIS | 0.117 | 54.71 | 0.1805 | 9.87 |
| DO | 5.15 | 5 | ICMR / BIS | 0.200 | 104.50 | 0.2767 | 28.91 |
| BOD | 3.95 | 5 | WHO / ICMR | 0.200 | 79.00 | 0.3068 | 24.24 |
| COD | 18.30 | 10 | WHO | 0.100 | 183.00 | 0.1534 | 28.09 |
| Alkalinity | 110.00 | 200 | BIS | 0.005 | 55.00 | 0.00767 | 0.42 |
| Hardness | 115.00 | 200 | WHO | 0.005 | 57.50 | 0.00767 | 0.44 |
| TDS | 240.00 | 500 | WHO / BIS | 0.002 | 48.00 | 0.00307 | 0.15 |
| TSS | 40.00 | 500 | WHO | 0.002 | 8.00 | 0.00307 | 0.02 |
| Nitrate | 0.25 | 50 | WHO | 0.020 | 0.50 | 0.03068 | 0.02 |

Overall WQI = 92.16

The Water Quality Index (WQI) condenses complex analytical data into a single value that subjectively represents overall water quality. This rating system enhances simplicity and public understanding. Various WQI models have been proposed in the literature, with comparative evaluations reported (Cude, 2001). Depending on its intended application, a WQI may be tailored for specific water bodies or designed as a general index applicable to all waters intended for human consumption.

Water Quality Index (WQI)

Inland ecosystems play a vital role in supplying drinking water, recharging groundwater, maintaining water quality, recycling nutrients, and removing toxins. In the present study, the calculated WQI of Begum Talab Lake was 92.16 (Table 3), indicating very good water quality. This score reflects conditions that are virtually free from threats or impairments, closely resembling natural or desirable levels (CCME, 2005) (Table-2). In comparison, Thakor et al. (2011) reported a WQI below 75 for Pariyej Lake, Gujarat; Bharamal and Korgaonkar (2014) recorded 75.82 for Palnekond Reservoir in Sawantwadi; and Alaka (2014) observed 76.27 in Bargaon Reservoir, Maharashtra.

Conclusion

The present study assessed the drinking water quality of Begum Talab Lake using physico chemical parameters and the Water Quality Index method. All the analysed parameters such as pH, dissolved oxygen, biological oxygen demand, chemical oxygen demand, alkalinity, hardness, total dissolved solids, total suspended solids, and nitrate were found within permissible limits of drinking water standards. The calculated Water Quality Index value of 92.16 indicates that the lake water falls under the “Very Good” category. This shows that the water quality is largely protected and close to desirable conditions, with only minor variations. Therefore, Begum Talab Lake water is suitable for drinking purposes with minimal treatment. However, regular monitoring and proper management are recommended to maintain its quality in the future.

Disclaimer (Artificial intelligence)

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

REFERENCE

- Alaka P A (2014) An assessment of water quality of Borgaon reservoir in Sangli district of Maharashtra, India. Int. Res. J. Environ. Sci. 3(5), 48-53.

- APHA (1998) Standard methods for the examination of water and waste water. (20 ed.). American Public Health Association, Washington D.C.

- Bharamal D L and Korgaonkar D S (2014) Determination of water quality index of Palnekond reservoir in Sawantwadi Taluka, Sindhudurg. Int. J. Curr. Microbiol. App. Sci. 3(6), 819-830.

- Bharathi, H. R., Manjappa, S., Suresh, T., & Suresh, B. (2016). Evaluation of water quality index of water bodies Channarayapatna taluk, Karnataka region, India. International Journal of Applied Sciences and Biotechnology, 4(4), 475-482.

- Bhat SA, Pandit AK (2014). Surface water quality assessment of Wular Lake, a ramsar site in Kashmir Himalaya, using discri minant analysis and WQI. J. Ecosyst. 2014

- CCME (Canadian Council of Ministers of the Environment) (2005) Standard Methods for the examinations of water and wastewater, American Public Health Association, Washington DC.

- Cude C G (2001) Oregon water quality index: A tool for evaluating water quality management effectiveness. J. American Water Research Assoc. 37, 125-137.

- Dubey R K, Hussain J, Malhotra N and Mehta A (2014) Ground water quality and water quality index of Dwarka district of National capital of India. I J R E T 3(4), 85-93.

- Garg R K, Rao R J and Saksena D N (2009) Water quality and conservation management of Ramsagar reservoir, Datia, Madhya Pradesh. J. Exp. Biol. 30(5), 909-916.

- Giriyappanavar B S and Patil R R (2013) Water Quality Assessment of Gullava Reservoir of Belgaum District Using NSF-WQI. I J S R. 2(4), 71-72.

- Gowda, K., & Sridhara, M. V. (2007). Conservation of tanks/lakes in the Bangalore metropolitan area. Management of Environmental Quality: An International Journal, 18(2), 137-151.

- Hulyal S B and Kaliwal B B (2011) Seasonal Variations in Physico Chemical Characteristics of Almatti Reservoir of Bijapur district, Karnataka State. I J E P 1(1), 58-67.

- Kangabam RD, Bhoominathan SD, Kanagaraj S, Govindaraju M (2017) Development of a water quality index (WQI) for the Loktak Lake in India. Appl Water Sci 7(6):2907–2918

- Khan, A.A., Gaur, R.Z., Mehrotra, I., Diamantis, V., Lew, B. & Kazmi, A.A. (2014). Performance assessment of different STPs based on UASB followed by aerobic post treatment systems. J Environ Health Sci Eng., https :// doi.org/10.1186/2052-336x-12-43

- Kodom K, Onoyinka AA. Mkude IT, Otu SA, Yeboah JO (2018). Physicochemical parameters assessment and seasonal variations of surface water quality of the volta lake in Ghana

- Kumar A Bahadur (2009) Physico-chemical Studies on the Pollution Potential of River Kosi at Rampur (India). World J. Agricult. Sci. 5(1), 1-4.

- Lodh R, Paul R, Karmakar B, Das MK (2014) Physicochemical stud ies of water quality with special reference to ancient lakes of Udaipur City, Tripura, India. Int J Sci Res Publ 4(6):1–9

- Ott WR (1978) Environmental Indices: Theory and Practice, Ann Arbor, MI (USA): Science Publishers Inc.

- Ouyang, Y. (2005). Evaluation of river water quality monitoring stations by principal component analysis. Water research, 39(12), 2621-2635.

- Pradhan, S. K., Patnaik, D., & Rout, S. P. (2001). Water Quality Index for the ground water around a phosphatic fertilizer plant. Indian Journal of Environmental Protection, 21(4), 355-358.

- Rajmohan, N (2003) Major correlation in ground water of Kancheepuram region, South India, Indian J. Environ. Hlth. 45(1): 1-5.

- Sawyer C H (1960) Chemistry for sanitary Engineers. McGraw Hill Book Co., New York.

- Sharma RC, Kumar R (2017) Water quality assessment of sacred gla cial Lake Satopanth of Garhwal Himalaya. India Appl. Water Sci. 7(8):4757–4764

- Sharma, P., Khitoliya, R.K. & Kumar, S. (2013). A comparative Sharma, P., Khitoliya, R.K. & Kumar, S. (2013). A comparative study of sewerage treatment plants with different technolo gies in the vicinity of Chandigarh city. J Environ Sci Toxi col Food Technol., 4(5): 113–121.

- Singh RP, Mangottiri V, Pandiyan B (2020) Understanding the vari ability in estimation of water quality of lakes and reservoirs. In IOP Conference Series: Materials Science and Engineering. 955(1):012085

- Singh, A. K., & Jayakumar, S. (2016). Water quality assessment of Kanwar Lake, Begusarai, Bihar, India. Imperial Journal of Interdisciplinary Research, 2(4), 793-803.

- Thakor S J, Bhoi D K, Dabhi H R, Pandya S N and Nikitaraj B C (2011) Water Quality Index (W.Q.I.) of Pariyej Lake Dist. Kheda – Gujarat. Curr. World Environ. 6(2), 225- 231.

- Tyor AK, Chawla D (2012). Survey and study of phytoplankton ecol ogy in Sukhna Lake, Chandigarh, (India)

- Usman A, Dube K, Shukla SP, Salaskar P, Prakash C, Sawant PB, Singh R (2018) Water quality index as a tool for assessment of status of an urban lake of Mumbai. Int J Curr Microbiol App Sci 7(04):520–533

- Verma P, Chandawat D, Gupta U, Solanki H (2012) Water quality analysis of an organically polluted lake by investigating differ ent physical and chemical parameters. Int J Res Chem Environ 2(1):105–111

- Wavde PN, Arjun B (2010) Groundwater quality assessment at Male gaon region of nanded in Maharashtra (India). J Environ Sci Eng 52(1):57–60