Original Research Article

Evaluation of Drinking Water Suitability of Mamadapur Lake, Vijayapura District, Based on the Water Quality Index

ABSTRACT

Freshwater resources are indispensable for domestic, agricultural, and industrial uses; however, rapid urbanisation, industrialisation, and population growth have led to significant deterioration in lake water quality worldwide. The Water Quality Index (WQI) has emerged as a reliable and integrative tool for evaluating physico-chemical parameters and determining the suitability of water bodies for human use and ecological sustainability. The present study evaluated the water quality of Mamadapur Lake located in Babaleshwar taluk of Vijayapuraa district, Karnataka, for drinking purpose using Water Quality Index. Surface water samples were collected at monthly intervals from July 2024 to June 2025 and analyzed for major physicochemical parameters. The mean pH value was 7.51±0.42, indicating a slightly alkaline nature within the permissible range of 6.5 to 8.5. Dissolved oxygen varied from 4.1 to 6.3 mg/L with an average of 5.33±1.56 mg/L. Biological oxygen demand ranged between 1.9 and 5.12 mg/L with a mean value of 3.12±2.28 mg/L. Chemical oxygen demand fluctuated from 19 to 23 mg per L. The average alkalinity and hardness were 96.14 mg/L and 106.15 mg/L, respectively. Total dissolved solids and total suspended solids recorded mean values of 223 mg/L and 34 mg/L, both within acceptable limits. Nitrate concentration remained low, averaging 0.17±0.12 mg/L. The overall Water Quality Index was calculated as 95.12, indicating excellent water quality. The results confirm that the lake water is suitable for drinking and domestic purposes.

Keywords: Water Quality Index (WQI); Physico-chemical parameters; Drinking water assessment; Freshwater lake monitoring

INTRODUCTION

Water is among the most abundant and vital natural resources on Earth (Kannel et al., 2007). Although it covers nearly 75% of the planet’s surface, only about 5% is available as freshwater for human use. Rainfall contributes to this supply, but much of it is lost through natural processes such as evaporation, transpiration, and evapotranspiration. A portion infiltrates the ground to recharge aquifers, while the rest is stored in lakes, ponds, and reservoirs (Nayek et al., 2017). High-quality water is essential not only for drinking and domestic needs but also for agriculture, industry, and commerce (Lyngdoh and Kayang, 2015). Lakes, in particular, play a crucial role in sustaining these activities. However, rapid urbanization and population growth have led to the degradation and disappearance of many lakes. Pollution further diminishes water quality, with contaminants seeping into groundwater and spreading to low-lying areas through percolation and influent seepage (Suthar et al., 2010). While industrialization and urbanization are inevitable drivers of national development, they also pose significant challenges to water sustainability (Mishra et al., 2009).

The Water Quality Index (WQI) is recognised as one of the most reliable and widely adopted tools for the comprehensive assessment of water quality (Ewaid et al., 2020). By integrating multiple physico-chemical and, in some cases, biological parameters into a single composite value, the WQI provides an accessible yet scientifically robust representation of overall water status. Cerqueira et al. (2020) examined the influence of land-use transformations on lotic systems in north-eastern Brazil and reported that rapid urbanisation has exerted a pronounced negative impact on the water quality of the Cachoeira River.

In recent years, advanced computational and data-driven modelling techniques have been increasingly applied to enhance the predictive accuracy and analytical efficiency of WQI estimation. For instance, Artificial Neural Networks (ANN), Multiple Linear Regression (MLR), and Adaptive Neuro-Fuzzy Inference System (ANFIS) have been utilised to estimate the WQI of the Yamuna River (Gaya et al., 2020). Similarly, Al-Adhaileh and Alsaade (2021) implemented ANFIS-based algorithms to predict WQI values for effective drinking water monitoring. In Malaysia, Yaseen et al. (2018) modelled the WQI of the Selangor River using hybrid intelligent approaches, integrating ANFIS with subtractive clustering, grid partitioning, and fuzzy C-means clustering techniques. Furthermore, Gazzaz et al. (2012) developed an ANN-based forecasting model for WQI computation in the Kinta River, demonstrating the applicability of soft computing techniques in river water quality assessment.

The widespread application of WQI in water quality investigations underscores its utility as a practical and effective decision-support tool. The index is formulated from a suite of water quality parameters, weighted according to their relative significance and regional standards, thereby enabling a simplified yet comprehensive evaluation of aquatic health. In this context, the primary objective of the present study was to assess the water quality of Mamadapur Lake using the WQI approach. The findings are expected to contribute to evidence-based management strategies, support the conservation of urban freshwater resources, and facilitate systematic monitoring and rejuvenation initiatives for sustainable lake ecosystems.

The concept of water quality extends beyond the characteristics of the water column to include the physical structure of the channel that sustains aquatic ecosystems. The Clean Water Act underscores the necessity of safeguarding the chemical, physical and biological integrity of the nation’s waters, thereby emphasising that effective assessment must address not only the condition of the water itself but also the habitats that support aquatic organisms. Nevertheless, accelerated urbanisation and associated anthropogenic activities have increasingly threatened this critical natural resource through various forms of pollution (Bharati et al., 2016).

The quality of surface water bodies, including lakes, reservoirs and rivers, is determined by a range of physico-chemical and biological parameters that collectively reflect their ecological status. In this context, the Water Quality Index (WQI) serves as a comprehensive indicator, providing an aggregated measure of water quality at a specific location and time based on multiple weighted parameters. Systematic monitoring of water quality is therefore indispensable for detecting pollution trends, assessing ecological risks, and evaluating the effectiveness of management and restoration strategies (Singh and Jayakumar, 2016; Sharma et al., 2025).  WQIs are generally classified into physico-chemical indices, derived from chemical and physical data, and biological indices, based on biological observations. In this study, WQI was calculated using hydrochemical parameters to assess the area’s water quality.

Mamadapur Lake, located in Babaleshwar taluk of Vijayapuraa, (16° 538419 N latitude and 75° 610672 E longitude) of Karnataka state, India, is a historic 16th-century waterbody built by Adil Shahi kings (Fig. 1). The Lake is known as ‘kere’, in the native language Kannada. The primary objective of the present study was to assess the water quality of Mamadapur lake to provide information on the physio-chemical parameters of Mamadapur Lake and its suitability for human consumption based on computed water quality index (WQI).

MATERIALS AND METHODS

Study area

The present investigation was conducted at Mamadapur Lake with the objective of evaluating its water quality in relation to drinking purposes. The lake, originally constructed during the Adil Shahi period, represents a significant historical water resource structure in the region. Historically, it possessed a maximum storage depth of approximately 7.6 metres, with a submergence area of nearly 864 acres and an irrigated command area of about 674 acres. These attributes highlight its former hydrological and socio-economic importance, particularly in supporting irrigation and local water demands. The lake was recently selected for rejuvenation under the Karnataka State government’s ambitious project. The water from this lake is mainly used for drinking and other domestic purposes.

Figure 1: Map of study area (Mamadapur Lake)

Composite surface water samples were collected at monthly intervals from Mamadapur Lake (Fig. 1) during the period July 2024 to June 2025. Water samples were collected in polypropylene bottles by fully immersing them in the water to prevent the introduction of air bubbles. A range of physicochemical parameters was analyzed, including pH, dissolved oxygen, biochemical oxygen demand, chemical oxygen demand, alkalinity, hardness, total dissolved solids, total suspended solids, and nitrate concentration. The measurement of pH was carried out directly at the sampling site to ensure accuracy using a digital pH meter (HANNA, model HI98186), whereas Dissolved oxygen and biochemical oxygen demand was determined using the Winkler method with separate 300 mL bottles prepared for incubation. Chemical Oxygen Demand (COD) was determined following the procedure described by Kotaiah and Kumaraswamy (1994). Total alkalinity was estimated by the titration method using 0.02 N standard sulphuric acid as the titrant, with phenolphthalein and methyl orange employed as indicators to determine the respective end points. Total hardness was analysed using the EDTA titrimetric method. Nitrate concentrations were measured by the brucine method with the aid of a UV–Visible spectrophotometer. All water samples were collected in appropriate containers and transported to the laboratory in dark ice boxes in order to minimise temperature fluctuations and prevent exposure to sunlight, thereby preserving sample integrity prior to analysis. Laboratory determination of the physico-chemical parameters was carried out in accordance with the protocols prescribed in the Standard Methods for the Examination of Water and Wastewater (APHA, 2017), ensuring accuracy, reliability and comparability of results. The Water Quality Index was subsequently calculated using the drinking water quality standards recommended by the World Health Organization, providing an integrated assessment of the suitability of Mamadapur Lake water for human consumption.

Calculation of WQI (Water Quality Index)

WQI=ÓQnwÓwn

Where:

- Qn = Water quality rating

Qn=100×Vn-ViSn-Vi

- Vn = Actual value of the parameter in the water sample

- Vi = Ideal value ((0 for all parameters except pH and DO which is 7.0 and 14.6 mg/l,       respectively)

- Sn = Standard value

- wn = Unit weight of the parameter

wn=KSn

- K = Constant, calculated as

K=11S1+1S2+…1Sn

Classification of WQI according to CCME (2005).

RESULTS AND DISCUSSION

Several investigations have evaluated lake water quality through the analysis of physicochemical parameters, as the overall condition of aquatic systems is largely determined by their chemical composition. Among these parameters, pH serves as a fundamental indicator of water suitability for domestic, agricultural, and ecological purposes. In Mamadapur Lake, the mean pH value was recorded at 7.51, consistently reflecting alkaline conditions throughout the study period. Similar observations were documented by Kodom et al. (2018) in a tropical lake in Ghana and by Singh et al. (2020) in Fateh Sagar Lake. The highest pH value of 8.1 was observed in September, while all recorded values remained within the desirable limits prescribed by the Bureau of Indian Standards, thereby indicating acceptable water quality in terms of acidity and alkalinity balance.

Dissolved oxygen (DO) levels are a key determinant of aquatic ecosystem health, shaping both the nature and distribution of aquatic organisms (Mequanent et al., 2022). Dissolved oxygen concentrations were fluctuated between 4.1 and 6.3 mg/L. Maximum levels were observed in February, coinciding with the summer season, whereas the lowest concentration was recorded in June. The current findings are consistent with those reported by Kodom et al. (2018) for Volta Lake, Ghana, but diverge from those of Kangabam et al. (2017) on Loktak Lake, Manipur, highlighting seasonal influences on oxygen dynamics. Biological oxygen demand, a parameter that reflects the degree of organic matter pollution and microbial activity (Kumar and Bahadur, 2009), exhibited values ranging from 1.9 mg/L in May to 5.12 mg/L in December. The elevated BOD levels during winter months suggest increased organic loading, possibly linked to anthropogenic inputs and reduced microbial degradation rates. Chemical oxygen demand, which provides a broader measure of oxidizable pollutants, varied between 19 and 23 mg/L. The lowest COD value was recorded in November, while the highest concentration occurred in June. These fluctuations point to seasonal variations in pollutant influx and organic matter decomposition, underscoring the dynamic nature of Mamadapur Lake’s water quality.

The average alkalinity of Mamadapur Lake was found to range between 75.0 and 112 mg/L, values that are consistent with earlier observations reported (Rahman et al., 2021). Alkalinity in freshwater systems arises from the release of CO₂ during the decomposition of organic matter (Gulista et al., 2021). It plays a crucial role in buffering aquatic systems against pH fluctuations, and the observed range indicates a stable environment supportive of aquatic life. Water hardness varied between 70 and 129 mg/L, with a mean value of 106 mg/L. Usman et al. (2018) reported elevated water hardness in Mumbai’s urban lakes during summer and reduced levels in winter, whereas Tyor & Chawla (2012) documented considerably lower hardness values in Sukhna Lake. Total dissolved solids (TDS) and total suspended solids (TSS) were recorded within the ranges of 20–320 mg/L and 9–69 mg/L, respectively. According to the Bureau of Indian Standards (BIS 10500, 2012), the desirable limit for TDS in drinking water is 500 mg/L, which may be extended up to 2000 mg/L in the absence of alternative sources (Dubey et al., 2014). The values observed in Mamadapur Lake therefore fall well within permissible limits, suggesting that the lake water is suitable for consumption from the perspective of dissolved and suspended solids. Nitrate concentrations were relatively low, ranging from 0.02 to 0.4 mg/L, with an average value of 0.17 mg/L. In contrast, Bhat and Pandit (2014) reported lower nitrate concentrations in Wular Lake, Kashmir during summer. Nitrogen levels in surface waters are elevated by decomposing plant matter and agricultural runoff. As a key indicator of organic pollution, nitrate originates from fertilizers, rainfall, crop residues, decaying organic waste, and septic systems (Khan et al., 2014). These concentrations are indicative of minimal nutrient enrichment and suggest limited anthropogenic input of nitrogen compounds. The low nitrate levels further reflect the absence of significant eutrophication risk during the study period, thereby supporting the ecological stability of Mamadapur Lake.

Water quality assessment of Mamadapur lake

Table 1: WQI classification by CCME (2005).

| WQI | Quality | Description |
| --- | --- | --- |
| 95–100 | Excellent | Water quality is protected with virtual absence of threat or impairment; conditions very close to natural or desirable levels. |
| 89–94 | Very good | Water quality is protected with a slight presence of threat or impairment; conditions close to natural or pristine levels. |
| 80–88 | Good | Water quality is protected with only minor degree of threat or impairment; conditions depart from natural or desirable levels. |
| 60–79 | Fair | Water quality is usually protected but occasionally threatened or impaired; conditions sometimes depart from natural or desirable levels. |
| 45–59 | Poor (Marginal) | Water quality is frequently threatened or impaired; conditions sometimes often depart from natural or desirable levels. |
| 0–44 | Very poor | Water quality is almost always threatened or impaired; conditions sometimes usually depart from natural or desirable levels (Unsuitable for drinking). |

Table 2 : WQI of Mamadapur lake water (all values except pH are in mg/l).

| Parameter | Actual Average Value | Standard Value | Recommended Agency | 1/Sn | Wn | Qn | QnWn |
| --- | --- | --- | --- | --- | --- | --- | --- |
| pH | 7.51 | 6.5–8.5 | ICMR / BIS | 0.1176 | 0.1805 | 55.56 | 10.03 |
| DO | 5.33 | 5 | ICMR / BIS | 0.2000 | 0.3068 | 105.00 | 32.21 |
| BOD | 3.12 | 5 | WHO / ICMR | 0.2000 | 0.3068 | 75.00 | 23.01 |
| COD | 19.65 | 10 | WHO | 0.1000 | 0.1534 | 180.00 | 27.61 |
| Alkalinity | 96.14 | 200 | BIS | 0.0050 | 0.00767 | 48.00 | 0.37 |
| Hardness | 106.15 | 200 | WHO | 0.0050 | 0.00767 | 50.00 | 0.38 |
| TDS | 223.00 | 500 | WHO / BIS | 0.0020 | 0.00307 | 45.00 | 0.14 |
| TSS | 34.00 | 500 | WHO | 0.1176 | 0.00307 | 6.50 | 0.02 |
| Nitrate | 0.17 | 50 | WHO | 0.2000 | 0.03068 | 0.20 | 0.01 |

Overall WQI = 95.12

The Water Quality Index (WQI) condenses complex analytical data into a single value that subjectively represents overall water quality. This rating system enhances simplicity and public understanding. Various WQI models have been proposed in the literature, with comparative evaluations reported (Cude, 2001). Depending on its intended application, a WQI may be tailored for specific water bodies or designed as a general index applicable to all waters intended for human consumption.

Water Quality Index (WQI)

In the present study, the calculated WQI of Mamadapur Lake was 95.12 (Table 2), indicating excellent water quality. This score reflects conditions that are virtually free from threats or impairments, closely resembling natural or desirable levels (CCME, 2005). In comparison, Thakor et al. (2011) reported a WQI below 75 for Pariyej Lake, Gujarat; Bharamal and Korgaonkar (2014) recorded 75.82 for Palnekond Reservoir in Sawantwadi; and Patil (2014) observed 76.27 in Bargaon Reservoir, Maharashtra.

Conclusion

The present study confirms that the physicochemical characteristics of Mamadapur Lake remain within acceptable limits for drinking water standards. The evaluated parameters indicate a stable and balanced aquatic environment with no significant signs of pollution or ecological stress. Seasonal variations were observed in certain parameters; however, these fluctuations did not adversely affect the overall water quality status. The calculated Water Quality Index classifies the lake water under the “Excellent” category, demonstrating that the water is largely free from major threats or impairments and closely reflects desirable natural conditions. Overall, Mamadapur Lake water is suitable for drinking and domestic purposes. Regular monitoring and appropriate management practices are recommended to maintain its quality and ensure long-term sustainability.

Disclaimer (Artificial intelligence)

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

REFERENCE

- American Public Health Association (APHA). (2017). "Standard Methods for the Examination of Water and Wastewater," 23rd ed., 9-38.

- Bharamal D L and Korgaonkar D S (2014) Determination of water quality index of Palnekond reservoir in Sawantwadi Taluka, Sindhudurg. Int. J. Curr. Microbiol. App. Sci. 3(6), 819-830.

- Bharathi, H. R., Manjappa, S., Suresh, T., & Suresh, B. (2017). Evaluation of water quality index of water bodies Channarayapatna taluk, Karnataka region, India. International Journal of Applied Sciences and Biotechnology, 4(4), 475-482.

- Bhat SA, Pandit AK (2014). Surface water quality assessment of Wular Lake, a ramsar site in Kashmir Himalaya, using discri minant analysis and WQI. J. Ecosyst. 2014

- Canadian Council of Ministers of the Environment (CCME) WQI (2005) Standard Methods for the examinations of water and wastewater. American Public Health Association, Washington, DC

- Cude C G (2001) Oregon water quality index: A tool for evaluating water quality management effectiveness. J. American Water Research Assoc. 37, 125-137.

- Dubey R K, Hussain J, Malhotra N and Mehta A (2014) Ground water quality and water quality index of Dwarka district of National capital of India. I J R E T 3(4), 85-93.

- Gulista, J., et al. (2021). Analysis of water quality using physico-chemical parameters in Paler Reservoir of Khammam District, Telangana, India. An International Journal of Advances in Pharmaceutical Sciences, 5(6), 2515-2518.

- I. Lyngdoh, H. Kayang, Physico-chemical and bacteriological characteristics of Umiam Lake, Meghalaya, India, c.

- Kangabam RD, Bhoominathan SD, Kanagaraj S, Govindaraju M (2017) Development of a water quality index (WQI) for the Loktak Lake in India. Appl Water Sci 7(6):2907–2918

- Khan, A.A., Gaur, R.Z., Mehrotra, I., Diamantis, V., Lew, B. & Kazmi, A.A. (2014). Performance assessment of different STPs based on UASB followed by aerobic post treatment systems. J Environ Health Sci Eng., https :// doi.org/10.1186/2052-336x-12-43

- Kodom K, Onoyinka AA. Mkude IT, Otu SA, Yeboah JO (2018). Physicochemical parameters assessment and seasonal variations of surface water quality of the volta lake in Ghana

- Kotaiah,B and Kumara Swamy,N., 1994,Enivornmental Engineering Laboratory manual, 1st ed., Charotar Publishing House, Anand, Gujaraj, 49-55.

- Kumar A Bahadur (2009) Physico-chemical Studies on the Pollution Potential of River Kosi at Rampur (India). World J. Agricult. Sci. 5(1), 1-4.

- M. Hmoud Al-Adhaileh, F. Waselallah Alsaade, Modelling and Prediction of Water Quality by Using Artificial Intelligence, Sustainability 13 (8) (2021) 4259.

- M.S. Gaya, S.I. Abba, A.M. Abdu, A.I. Tukur, M.A. Saleh, P. Esmaili, N.A. Wahab, Estimation of water quality index using artificial intelligence approaches and multi-linear regression, Int. J. Artif. Intell. ISSN 2252 (2020) 8938.

- Mequanent, D., Mingist, M., Getahun, A., Anteneh, W., Getnet, B., Birie, S., 2022. The investigation of zooplankton community in the newly formed Ribb reservoir, Ethiopia: the tropical highland reservoir. Heliyon 8, e10533. https://doi.org/ 10.1016/j.heliyon.2022.e10533.

- Mishra, A., Mukherjee, A., & Tripathi, B. D. (2009). Seasonal and temporal variations in physico-chemical and bacteriological characteristics of River Ganga in Varanasi

- N.M. Gazzaz, M.K. Yusoff, A.Z. Aris, H. Juahir, M.F. Ramli, Artificial neural network modelling of the water quality index for Kinta River (Malaysia) using water quality variables as predictors, Mar. Pollut. Bull. 64 (11) (2012) 2409 2420

- P.R. Kannel, S. Lee, Y.S. Lee, S.R. Kanel, S.P. Khan, Application of water quality indices and dissolved oxygen as indicators for river water classification and urban impact assessment, Environ. Monit. Assess. 132 (1) (2007) 93–110.

- Patil A A (2014) An assessment of water quality of Borgaon reservoir in Sangli district of Maharashtra, India. Int. Res. J. Environ. Sci. 3(5), 48-53.

- Rahman, A., Jahanara, I., & Jolly, Y. N. (2021). Assessment of physicochemical properties of water and their seasonal variation in an urban river in Bangladesh. Water Science and Engineering, 14(2), 139-148.

- S. Nayek, I.H. Choudhury, S. Roy, Water quality assessment of two constructed lakes (Aritar lake & Chaya taal lake) in Sikkim during winter & summer season, Int. J. Adv. Res 5 (7) (2017) 1411–1417. Materials Today: Proceedings xxx (xxxx) xxx

- S. Suthar, J. Sharma, M. Chabukdhara, A.K. Nema, Water quality assessment of river Hindon at Ghaziabad, India: impact of industrial and urban wastewater, Environ. Monit. Assess. 165 (1) (2010) 103–112.

- S.H. Ewaid, S.A. Abed, N. Al-Ansari, R.M. Salih, Development and Evaluation of a Water Quality Index for the Iraqi Rivers, Hydrology. 7 (3) (2020) 67.

- Singh RP, Mangottiri V, Pandiyan B (2020) Understanding the vari ability in estimation of water quality of lakes and reservoirs. In IOP Conference Series: Materials Science and Engineering. 955(1):012085

- Singh, A. K., & Jayakumar, S. (2016). Water quality assessment of Kanwar Lake, Begusarai, Bihar, India. Imperial Journal of Interdisciplinary Research, 2(4), 793-803.

- T.C. Cerqueira, R.L. Mendonça, R.L. Gomes, R.M. de Jesus, D.M.L. da Silva, Effects of urbanization on water quality in a watershed in north-eastern Brazil, Environ. Monit. Assess. 192 (1) (2020) 1–17.

- Thakor S J, Bhoi D K, Dabhi H R, Pandya S N and Nikitaraj B C (2011) Water Quality Index (W.Q.I.) of Pariyej Lake Dist. Kheda – Gujarat. Curr. World Environ. 6(2), 225- 231.

- Tyor AK, Chawla D (2012). Survey and study of phytoplankton ecol ogy in Sukhna Lake, Chandigarh, (India)

- Usman A, Dube K, Shukla SP, Salaskar P, Prakash C, Sawant PB, Singh R (2018) Water quality index as a tool for assessment of status of an urban lake of Mumbai. Int J Curr Microbiol App Sci 7(04):520–533

- Z.M. Yaseen, M.M. Ramal, L. Diop, O. Jaafar, V. Demir, O. Kisi, Hybrid adaptive neuro-fuzzy models for water quality index estimation, Water Resour. Manag. 32 (7) (2018) 2227–2245.

- Sharma, A., Kumar, R., Devi, S., & Sharma, S. (2025). Assessment of Drinking Water Quality and Water Quality Index (WQI) of the Newly Formed Municipal Corporation Town, Palampur, Himachal Pradesh, India. Asian Journal of Environment & Ecology, 24(6), 51-60.