Role of Microorganisms in Oil Reservoir Souring and Its Mitigation Strategies

Abstract

Petroleum microbiology offers substantial environmental and industrial benefits by harnessing the natural capabilities of microbes to interact with crude oil and refined petroleum products. Microorganisms are found worldwide in oil reservoirs, where they degrade the oil and alter its quality. Oil reservoirs are challenging environments for microorganisms due to their high toxicity, hydrophobicity, low water activity, and extreme conditions, including high temperature, salinity, and pressure. Despite these harsh factors, oil reservoirs host a diverse range of bacteria and archaea, including fermenters, acetogens, methanogens, and organisms that reduce sulfate, nitrate, and iron. It is crucial to understand the microbiology of oil reservoirs, as oil remains one of the most vital resources for industry and energy. Sulfate-reducing bacteria (SRB) convert sulfate to sulfide, a hazardous and corrosive compound that impacts metallurgy, capital and operating costs, and operational safety. Reservoir souring is the process where a reservoir that previously did not contain any hydrogen sulfide (H2S) begins to produce sour fluids containing H2S due to microorganisms. The genetic, enzymatic, and physiological aspects of the sulfate-reduction process have been extensively studied in pure cultures in laboratory settings. Furthermore, advancements in DNA sequencing have identified new groups of microorganisms, including archaea, that can contribute to reservoir souring. Reservoir souring is primarily caused by sulfate reducing microorganisms, which leads to the formation of H2S in oil reservoirs during water flooding, although both biotic and abiotic processes are involved. This review discusses the occurrence of reservoir souring due to specific types of sulfate-reducing microorganisms and outlines strategies for managing this issue through the injection of various chemicals. In addition, the manuscript provides useful insights that can guide future research and improve reservoir management practices in the oil and gas industry.

Keywords: biological souring; oil reservoir; microorganisms; corrosion.

Introduction

Microbial communities and their activities in petroleum reservoirs are crucial for understanding the biogeochemical processes and organisms that operate under the harsh conditions of pressure and temperature. Additionally, they are crucial for developing new technologies and methods for bioremediation of petroleum-contaminated areas (Hosseini et al., 2025; Koshlaf, Ball, 2017), managing reservoir souring (Szymczak et al., 2020), and addressing corrosion in oil production systems (Shah, Muthalif,2025), as well as enhancing microbial energy recovery (Iwogbemi, 2025). The investigation of microbial metabolisms in deep underground environments is best performed in petroleum reservoirs (Ramy et al., 2025; Trutschel et al., 2022), which can be seen as a natural bioreactor. The geochemistry of oil reservoir habitats is significantly influenced by the activities of microorganisms. The relationship between microbial metabolic processes and carbon turnover, element cycling, carbon capture, utilization, and sequestration in petroleum reservoirs is interesting (Qiao et al., 2025; Wu et al., 2024). This includes aspects such as microbial enhanced oil recovery (MEORand the prevention of microbiologically influenced corrosion in crude oil fields (Odeyemi, Alaba, 2024). The microbial enhanced oil recovery technique (MEOR) is vital for extracting residual oil from reservoirs that are close to their economic limits, especially those with high water cut, low permeability, and heavy oil that traditional methods cannot economically recover (Das et al., 2025). Research on microbial enhanced oil recovery (MEOR) has extensively investigated the dominant microbial communities present in petroleum reservoirs, the metabolic products that can enhance oil recovery, and the methods applicable to varying reservoir conditions (Cabral et al., 2021). MEOR demonstrates significant promise in several areas, including microbial water flooding, modulation of microbial permeability profiles, and the microbial huff-and-puff technique, and is widely regarded as a cost-effective and environmentally sustainable approach to oil recovery (Ariadji et al., 2017).

In microbial water flooding, the primary mechanisms involve the reduction of oil–water interfacial tensions by biosurfactants and the alteration of the wettability of oil-bearing rocks. Modification of the microbial permeability profile is primarily achieved through biomass accumulation in high-permeability zones of heterogeneous reservoirs, which redirects fluid flow and enhances oil displacement efficiency (Zhang et al., 2012). Both culture-dependent and culture-independent studies have identified diverse microorganisms capable of converting CO2 to CH4 under a range of temperature conditions. Functional gene-based analyses of microbial communities in reservoirs further confirm the potential of these CO2-to-methane conversion pathways, highlighting opportunities for energy recovery from CO2 (Kennes, 2018).

Laboratory and field investigations have focused on microbial populations and methanogenic metabolic processes in petroleum reservoirs. Critical components of methanogenesis include microorganisms, electron donors, and the thermodynamic and kinetic characteristics of these reactions, with the availability of electron donors often representing the most limiting factor (Okoro et al., 2026). Sulfate-reducing microorganisms (SRMs) are widely distributed in subsurface petroleum reservoirs and associated oil-well pipelines. These bacteria can be anaerobic, heterotrophic, autotrophic, or partially tolerant to oxygen, enabling them to adapt to diverse environmental conditions such as temperature, salinity, and pressure (Zhou et al., 2025).

Microbial communities in petroleum reservoirs are dominated by taxa including Proteobacteria, Nitrospira, Firmicutes, Thermodesulfobacteria, Euryarchaeota, and Crenarchaeota. SRMs play key roles in the biogeochemical cycling of carbon, nitrogen, and sulfur and are central to strategies for petroleum reservoir exploitation (Demin et al., 2024). However, they can also contribute to microbially induced corrosion (MIC), reservoir souring, and crude oil biodegradation. These processes occur through the oxidation of hydrogen (H2) or organic substrates—including hydrocarbons, volatile fatty acids, alcohols, and aromatic compounds—via sulfate reduction, resulting in the production of hydrogen sulfide (H2S) (Khan et al., 2021).

To mitigate reservoir souring and biocorrosion, various inhibitory strategies, including biocide injection and the application of nitrate or perchlorate, have been employed. The effectiveness of these interventions is often contingent upon the specific composition and functional mechanisms of the SRM populations in different reservoirs (Labena et al., 2023). This manuscript provides a focused overview of biological souring, mitigation strategies, and the microorganisms responsible for these processes in petroleum reservoirs.

- Reservoir souring

Reservoir souring is a significant and unacceptable threat that must not be overlooked in the oil and gas industry. Its impacts on operational efficiency and environmental safety are serious and demand immediate attention. The process in which microorganisms, primarily sulfate-reducing bacteria (SRB), convert sulfate ions in formation water into hydrogen sulfide (H2S) gas is known as reservoir souring, or microbial-induced corrosion (MIC). In the oil and gas industry, H2S presents significant challenges due to its highly corrosive nature, toxicity, and unpleasant odor (Harshita et al., 2026; Priya et al., 2025; Dolfing, Hubert, 2017). Reservoir souring refers to the biological production of hydrogen sulfide (H2S) by sulfate-reducing bacteria (SRB) in oil reservoirs, usually triggered by seawater flooding. Since H2S is both toxic and corrosive, reservoir souring presents significant economic and environmental challenges. As a result, extensive research has been conducted over the past few decades to understand the biological mechanisms underlying reservoir souring (Vance, Thrasher, 2007). To predict field-scale reactions based on laboratory findings, researchers often utilize reservoir souring models. These investigations into souring are conducted using three types of inoculums: backflow water from an injection well, produced water, and proxy inoculums. Researchers often utilize reservoir souring models to predict field-scale responses based on laboratory findings. Three types of inoculums used in these souring studies include generated water, backflow water from an injection well, and proxy inoculums.

When a reservoir containing production and injection wells is injected with seawater that is colder than the reservoir temperature, fluid samples can be collected from a few accessible sampling locations. These samples represent the fluid in the reservoir and are used in reservoir sourcing experiments. The collected water samples offer valuable insights into how the indigenous microbiome of the reservoir interacts with the injected water microbiome. However, since the conditions in the well can differ significantly from those in the aquifer, it has been shown that there is a notable difference between the biota found in groundwater and the samples obtained from groundwater wells.

Symptoms

of reservoir

souring

souring

Fig 1: Symptoms of reservoir souring.

- Causes of reservoir souring

In the oil and gas industry, reservoir souring is a common and costly issue, especially during secondary recovery processes. The primary cause of this problem is the activity of sulfate-reducing bacteria (SRB) and sulfate-reducing archaea (SRA) (Mahajan et al., 2025). These microorganisms break down sulfate ions in water, which leads to the production of hydrogen sulfide (H2S) (Sindi et al, 2025; Vance, Thrasher, 2005; Vance, Thrasher, 2007) (Fig 1).

The generation of hydrogen sulfide (H2S), a hazardous and corrosive gas, from previously sweet (H2S-free) oil and gas reserves is known as reservoir souring. This process is often associated with saltwater injection, a technique commonly used in secondary oil recovery to enhance oil production and maintain reservoir pressure. The introduction of seawater, which is rich in sulfate, can stimulate the growth of sulfate-reducing microorganisms (SRMs) that produce H2S.

- Strategies for Managing Reservoir Souring by biocide and nitrate injection

Biocides are substances specifically designed to inhibit the growth of sulfate-reducing bacteria and other microorganisms associated with souring. Regular injection of biocide into the reservoir effectively controls microbial activity and prevents souring. Nitrate acts as an alternate electron acceptor for sulfate reducing bacteria (SRB), diverting their metabolic pathways away from H2S production. Injecting nitrate into the reservoir can also decrease the generation of souring agents. (Okpala et al., 2018) In addition, monitoring and Management of Reservoirs is an important task to continuously monitor reservoir conditions, including water chemistry and microbiological activity. Rapid action, such as adjusting pH levels or employing water injection techniques, may be taken upon early detection of souring to prevent or mitigate its effects (Dabral et al., 2025; Carlson, Hubert, 2019; Kaster et al., 2014). In the oil and gas sector, sourcing reservoirs remains a challenging issue, especially in older assets where hydrogen sulfide (H2S) and microorganisms from the reservoir can significantly compromise the integrity of producing wells and equipment. Nitrate treatment is commonly used to manage sulfate-reducing bacteria (SRB) activity in reservoirs; however, it is not always effective due to insufficient understanding of the subsurface conditions and the increasing quantities required over time. Designing an effective nitrate treatment plan requires a comprehensive understanding of the microbial reactions involved, particularly those related to the diverse microbial communities within the reservoir. It is also important to understand how nitrate inhibits the souring process. Nitrate promotes the growth of nitrate-reducing bacteria that can outcompete sulfate-reducing bacteria, thereby decreasing hydrogen sulfide production and alleviating souring. This integrated knowledge is essential for optimizing treatment strategies and ensuring the integrity of the reservoir (Gutierrez et al., 2009)  (Fig 2).

Managing

reservoir

souring

Fig. 2:  Strategies for Managing Reservoir Souring.

- Reservoir Souring and the Formation of Hydrogen Sulfide

A range of organic compounds, including alcohols, polar aromatic hydrocarbons, and saturated hydrocarbons, can reduce aqueous sulfate derived from saltwater, formation pore water, or the degradation of solid calcium sulfate at temperatures exceeding 250 °C. Elevated concentrations of hydrogen sulfide (H2S) in reservoir fluids can result from thermochemical sulfate reduction (TSR), a process driven by thermal redox reactions. Experimental studies have investigated the reaction pathways, potential products, and the influence of variables such as temperature, oxidant type, sulfur species, metal cations, and pH on the rate of TSR. This phenomenon is widely documented in petroleum geochemistry. H2S formation during TSR occurs via thermolysis of aliphatic organosulfur compounds, such as tetrahydrothiophene, as well as aromatic thiophene-type compounds present in heavy oils (Guo & Liu, 2024). Certain petroleum reservoirs are classified as sour due to non-biogenic processes, which include the thermophilic decomposition of sulfur-containing hydrocarbons, pyrite dissolution, and TSR. The efficiency and extent of these mechanisms are influenced by several factors, including reservoir rock type, oil composition, thermal maturity, elevated temperatures, and aquathermolysis during thermal recovery operations.

- Reservoir management and monitoring

Reservoir management is a critical determinant of the economic viability of petroleum operations, as it directly affects production rates, recovery factors, and the long-term value of assets. Decisions regarding drilling, completion, and production optimisation involve substantial capital and operating expenditures and carry significant safety and environmental implications. In the context of aging assets, volatile markets, and increasingly stringent regulatory requirements, timely and accurate reservoir analysis has become an operational imperative. The overarching objective of reservoir management is to maximise hydrocarbon recovery while minimising capital outlay and operational costs. This process is integrated, dynamic, and continuous, requiring the efficient coordination of interdisciplinary human, technical, and financial resources (Satter & Iqbal, 2016).

Additionally, it requires frequent adjustments to management plans based on the performance of the reservoir. Furthermore, the reservoir management process includes economic, legal, social, and environmental aspects. Reservoir management is an ongoing process that involves the continuous collection and analysis of data in real-time to enhance procedures and improve well efficiency. Commonly gathered information includes well rates, the water-to-oil ratio, bottom hole pressure, and the composition of produced water (Thakur, 1996). As technology continues to advance, particularly in areas like artificial intelligence, machine learning, and the internet, reservoir monitoring is likely to experience significant innovation in the future (Aliev et al., 2023; Ma et al., 2023). These technologies are expected to enhance real-time analysis and automate data collection, leading to greater accuracy in managing production processes and providing deeper insights (Ge et al., 2025). Industry experts suggest that incorporating AI could enhance predictive models, leading to better reservoir management and proactive problem-solving. As this technology continues to evolve, operators will be more equipped to address emerging challenges and take ad avantage of opportunities for sustainable resource utilization (Shao et al., 2025).

- Importance of Reservoir management for sustaining oil production

Reservoir management plays a pivotal role in the oil and gas industry, serving as a cornerstone for sustaining future oil production. Through the optimisation of hydrocarbon recovery, reduction of operational costs, prolongation of oilfield lifespan, and achievement of sustainability objectives, effective reservoir management enables companies to ensure a reliable and responsible supply of oil to meet global energy demands (Guo & Liu, 2024). The recoverable volume of oil from a reservoir can be enhanced through strategic interventions such as enhanced oil recovery (EOR) (Das et al., 2025). EOR techniques, which involve the injection of gases or chemicals into the reservoir, mobilise otherwise unrecoverable oil and facilitate its extraction. In addition to increasing total production, EOR can also extend the productive life of a reservoir. Effective reservoir management further contributes to the reduction of operating costs associated with oil production. By continuously monitoring reservoir performance and optimising production processes, companies can identify opportunities to reduce expenditures. For example, operational efficiencies can be achieved by minimising the volume of water or gas injection or by optimising artificial lift systems to decrease energy consumption (Ma et al., 2025). Prolonging the life of oilfields through strategic reservoir management ensures sustained production capacity, thereby supporting a stable energy supply even amid rising global demand. Techniques such as pressure management, production optimisation, and EOR implementation collectively maximise the extractable oil from a given reservoir.

Fig 3: Summary of the main objectives of the reservoir management activities.

- Key elements of effective reservoir management

Optimizing oil and gas production in reservoirs to ensure maximum recovery and sustainability is a critical aspect of the oil and gas industry. This process involves several key tasks, such as monitoring well performance, predicting reservoir behavior, and implementing strategies to maximize production (Rajput and Pathak, 2025).

The key elements of reservoir management are intricate and require the integration of various fields, including production engineering, geology, geophysics, and reservoir engineering as follows :-

a)	Reservoir characterization involves gathering information to understand the key features of a reservoir, such as rock type, permeability, porosity, and hydrocarbon volume. A comprehensive understanding of these characteristics is essential for developing effective management plans. This information plays a crucial role in the process. For instance, utilizing seismic data can help identify the locations of gas and oil deposits within the reservoir (Nabawy and Bassem, 2025; Shiba and Vijay, 2023).

b)	Reservoir Monitoring: This involves the continuous monitoring and evaluation of the reservoir's performance, such as the production rate, pressure, and temperature. The data obtained from monitoring can help in identifying changes in the reservoir's behavior, such as the depletion of oil and gas reserves (Ge et al., 2025).

c)	Reservoir simulation involves modeling the behavior of a reservoir under different conditions using numerical methods. This process can help predict future reservoir behavior, including the remaining oil and gas reserves and expected production rates. For example, reservoir simulation can be used to identify the optimal location for drilling wells to maximize output (Saradva and Harshil, 2025).

d)	The process of developing and implementing strategies to optimize the extraction of gas and oil from a reservoir is referred to as reservoir management. This may include secondary and tertiary recovery methods, such as gas injection and water flooding. These techniques are designed to enhance the flow of gas and oil to the surface and increase the pressure within the reservoir (Holm et al.,2025).

e)	To maximize the production of oil and gas, effective reservoir management involves complex elements that require the integration of multiple disciplines. Gaining a thorough understanding of the reservoir and implementing efficient management techniques can be enhanced through reservoir characterization, monitoring, simulation, and management practices (Paulraj et al., 2025).

- Latest development of reservoir souring

Research conducted in laboratory settings has thoroughly examined the genetic, enzymatic, and physiological aspects of sulfate reduction using pure cultures. Moreover, DNA sequencing has identified novel microbial taxa, including certain archaea, that can contribute to reservoir souring. Although many recent advancements in microbial management and detection have emerged, many of the techniques still in use today are over a century old. Alongside established methods, we now consider new and emerging mitigation technologies that can be used to prevent reservoir souring. In managing microbial reservoir souring, both modeling and prediction are essential yet often underutilized tools. Models must take into account partitioning and mineral scavenging, in addition to microbial hydrogen sulfide (H2S) production, to be truly predictive. The rise of "big data," fueled by the increased integration of sensors in digital oil fields and advancements in DNA sequencing through next-generation sequencing (NGS), presents a rare opportunity to develop and refine microbial reservoir souring models. Consequently, we analyze various reservoir souring models and explore their potential applications (Holm et al.,2025) (Fig 3).

- Microbial reservoir souring mechanisms

Reservoir souring is caused by both biological and non-biological processes. However, the primary mechanism for the formation of hydrogen sulfide (H2S) in oil reservoirs, especially as a result of water flooding, is the reduction of sulfate by microorganisms (Holm et al.,2025; Vance and Thrasher, 2005). Several factors influence this process, including the water supply, sulfate concentrations, available carbon sources, other nutrients, trace metals, temperature, pressure, salinity, and the presence of a functional sulfate-reducing population (SRP). Reservoir souring can lead to significant economic impacts, especially considering an anticipated 2% cost increase for sour service materials at the design stage of any project. If sour service materials need to be retrofitted, the costs can increase dramatically (Vance and Thrasher, 2005). These expenses are substantial, particularly since many oil and gas megaprojects exceed $1 billion USD in total cost. Additionally, the consequences of reservoir souring contribute to the already high costs associated with corrosion in the oil and gas industry. It is estimated that corrosion will cost the upstream sector approximately $1.4 billion annually in the United States alone.

- Regulation of sulfide accumulation in high-temperature oil reservoirs

The rich availability of electron donors, along with high salinity, elevated temperatures, and significant pressure, creates unique environments for microorganisms within oil reserves (Pannekens et al., 2019). In the presence of sulfate, processes such as fermentation, methanogenesis, and sulfate reduction play a crucial role in shaping microbial communities in pristine oil reserves. While nitrate and oxygen are typically absent from subterranean oil reserves, they can be introduced during secondary oil recovery when saltwater is injected into the reservoir to maintain stable pressure (Gieg, 2018). In the Danish section of the North Sea, the reservoir rock for Halfdan oil production is located 2000 meters below the seafloor and has a temperature exceeding 80 degrees Celsius (Gittel et al., 2009). The temperature of the reservoir decreases due to water injection, creating a temperature gradient from the injection site to the areas where oil and water are produced. As the injected cold saltwater, which has a temperature of 6 to 8 °C, descends into the reservoir, it warms by 2 to 3 °C for every 100 meters of depth. By the time it reaches a depth of 2000 meters. The water injection lowers the reservoir temperature, creating a temperature gradient from the injection site to the oil and water production zones. As the injected water moves through the reservoir, it heats up to nearly the formation temperature of 80 °C. After passing through the reservoir, the oil and water mixture gradually cools to 60 °C as it rises to the surface.

In oilfields, nitrate is commonly used as a souring mitigation agent (Johnson et al., 2017). Field studies have shown that injecting 5 mM of nitrate can reduce sulfide buildup by 50–80% (Telang et al., 1997; Jeneman et al., 1999). There are four potential ways in which nitrate helps to prevent the buildup of sulfide in oil reserves. First, nitrate-reducing bacteria gradually convert nitrate into either ammonia or dinitrogen. This process leads to a temporary accumulation of nitrite in the environment, which may inhibit the growth of SRM. Research by Greene et al. (2003) indicates that the activity of nitrite reductase in these strains is negatively correlated with how effectively nitrite suppresses the activity of SRM in pure cultures.

ii) Some SRM may preferentially utilize nitrate and nitrite as electron acceptors instead of sulfate. Consequently, their presence can inhibit sulfate-reducing processes (Marietou 2016).

iii) Competitive exclusion occurs when heterotrophic nitrate-reducing microbes outcompete SRM for the same electron donors (Hubert and Voordouw, 2007).

iv)  Lastly, nitrate injection may enhance the development and activity of autotrophic nitrate-reducing, sulfide-oxidizing bacteria (NR-SOB), which inhibit sulfate-reducing microorganisms (SRM) by producing nitrite and preventing sulfide accumulation in the reservoir, thereby avoiding souring and corrosion (Greene et al., 2003).

Conclusion

Effective reservoir monitoring is essential for generating oil and gas. Operators can optimize production while maintaining environmental responsibility by adopting advanced monitoring techniques and technologies. Embracing these innovations is crucial as industry practices evolve to ensure the long-term viability and efficiency of reservoir management. Seeking guidance from service providers can help you successfully integrate these technologies into your business processes. In conclusion, the ongoing advancements in reservoir engineering are driving significant improvements in oil and gas recovery. The combination of enhanced recovery techniques, advanced modeling tools, machine learning, and real-time monitoring is leading to more efficient and sustainable production. As these technologies continue to evolve, the oil and gas industry will be better equipped to meet the growing global demand for energy while minimizing its environmental impact.

Disclaimer (Artificial intelligence)

The authors hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

References

- Aliev TA, Guluyev GA, Rzayev AsH, Pashayev FH, Rzayeva NE, Gadimov RM. (2023). A technology for monitoring the onset of malfunctions in oil field reservoir pressure maintenance equipment. Informatics and Control Problems 43 (1): 1-12. https://doi.org/10.54381/icp.2023.1.01.

- Ariadji, T., Astuti, D. I., Aditiawati, P., Purwasena, I. A., Persada, G. P., Soeparmono, M. R., Amirudin, N. H., Ananggadipa, A. A., Sasongko, S. Y., Abqory, M. H., Ardianto, R. N., Subiantoro, E., and G. H. Aditya. "Microbial Huff and Puff Project at Mangunjaya Field Wells: The First in Indonesia Towards Successful MEOR Implementation." Paper presented at the SPE/IATMI Asia Pacific Oil & Gas Conference and Exhibition, Jakarta, Indonesia, October 2017. doi: https://doi.org/10.2118/186361-MS.

- Cabral, L., Giovanella, P., Pellizzer, E. P., Teramoto, E. H., Kiang, C. H., & Sette, L. D. (2021). Microbial communities in petroleum-contaminated sites: Structure and metabolisms. Chemosphere, 286, 131752. https://doi.org/10.1016/j.chemosphere.2021.131752.

- Carlson, H.K., Hubert, C.R.J. (2019). Mechanisms and Monitoring of Oil Reservoir Souring Control by Nitrate or Perchlorate Injection. In: McGenity, T. (eds) Microbial Communities Utilizing Hydrocarbons and Lipids: Members, Metagenomics and Ecophysiology . Handbook of Hydrocarbon and Lipid Microbiology . Springer, Cham. https://doi.org/10.1007/978-3-319-60063-5_17-1.

- Dabral R; Sandhu H.A.S.; Trivedi M. Claudia Cherubini (2025). Optimal operation of reservoir for sustainable water management using multi-criteria decision-making approach. Journal of Hydroinformatics (2025) 27 (5): 878–899. https://doi.org/10.2166/hydro.2025.006.

- Das, A., Das, N., Pandey, P. et al. Microbial enhanced oil recovery: process perspectives, challenges, and advanced technologies for its efficient applications and feasibility. Arch Microbiol 207, 106 (2025). https://doi.org/10.1007/s00203-025-04307-1.

- Demin KA, Prazdnova EV, Minkina TM, Gorovtsov AV. (2024). Sulfate-reducing bacteria unearthed: ecological functions of the diverse prokaryotic group in terrestrial environments. Appl Environ Microbiol. 90(4):e0139023. Epub 2024 Mar 29. PMID: 38551370; PMCID: PMC11022543. https://doi:10.1128/aem.01390-23.

- Dolfing J, Hubert CRJ. (2017). Using Thermodynamics to Predict the Outcomes of Nitrate-Based Oil Reservoir Souring Control Interventions. Front Microbiol. 19;8:2575. PMID: 29312252; PMCID: PMC5742191. https://doi:10.3389/fmicb.2017.02575.

- Ge, L., Wei, Y., Min, C., Yang, Q., & Tian, G. (2025). Research status and prospect of intelligent wells reservoir monitoring technology. Nondestructive Testing and Evaluation, 1–43. https://doi.org/10.1080/10589759.2025.2528931.

- Ge, L., Wei, Y., Min, C., Yang, Q., & Tian, G. (2025). Research status and prospect of intelligent wells reservoir monitoring technology. Nondestructive Testing and Evaluation, 1–43. https://doi.org/10.1080/10589759.2025.2528931.

- Gieg LM (2018) Microbial communities in oil shales, biodegraded andheavy oil reservoirs, and bitumen deposits. In: McGenity T (ed)Microbial communities utilizing hydrocarbons and lipids: members,Metagenomics and Ecophysiology. Handbook of Hydrocarbon andLipid Microbiology. Springer, Cham.

- Gittel A, Kofoed MV, Sørensen KB, Ingvorsen K, Schramm A (2012) Succession of Deferribacteres and Epsilonproteobacteria through anitrate-treated high-temperature oil production facility. Syst ApplMicrobiol 35:165–174

- Greene EA, Hubert C, Nemati M, Jenneman GE, Voordouw G (2003), Nitrite reductase activity of sulphate-reducing bacteria prevents their inhibition by nitrate-reducing, sulphide-oxidizing bacteria. Environ Microbiol 5:607–617.

- Greene EA, Hubert C, Nemati M, Jenneman GE, Voordouw G (2003). Nitrite reductase activity of sulphate-reducing bacteria prevents their inhibition by nitrate-reducing, sulphide-oxidizing bacteria. Environ Microbiol 5:607–617.

- Guo, Y., & Liu, L. (2024). Reservoir Control Operations and Water Resources Management. Water, 16(20), 3000. https://doi.org/10.3390/w16203000.

- Gutierrez, O., Park, D., Sharma, K.R. and Yuan, Z. (2009) Effects of long-term pH elevation on the sulfate-reducing and methanogenic activities of anaerobic sewer biofilms. Water research 43, 2549-2557.

- Harshita, Chandra, S., Banerjee, S., Pandit, S., Jadhav, D. A., Gill, H. S., … Rajeev, M. (2026). Sulfate-reducing bacteria as natural bioelectrochemical catalysts: unveiling their role in extracellular electron transfer. Biocatalysis and Biotransformation, 1–24. https://doi.org/10.1080/10242422.2026.2615663.

- Holm, Casper & Kringelum, Louise & Anand, Amitabh. (2025). Creating effective strategy implementation:  a systematic review of managerial and organizational levers. Review of Managerial Science.  20. 673-705. https://10.1007/s11846-025-00880-3.

- Hosseini, S., Sharifi, R. & Habibi, A. (2025). Efficient bioremediation of crude oil contaminated soil by a consortium of in-situ biosurfactant producing hydrocarbon-degraders. Sci Rep 15, 19852. https://doi.org/10.1038/s41598-025-05035-8. ISSN 0048-9697, https://doi.org/10.1016/j.scitotenv.2025.180226.

- Hubert C, Voordouw G (2007) Oil field souring control by nitrate reducing Sulfurospirillum spp. that outcompete sulfate-reducing bacteria for organic electron donors. Appl Environ Microbiol 73: 2644–2652.

- Iwogbemi A.O. (2025) Microbial Enhanced Oil Recovery (MEOR): Innovations in Sustainable Oil Extraction, International Journal of Petroleum and Gas Engineering Research, 8 (1), 1-1, https://doi.org/10.37745/ijpger.17v8n1112.

- Jenneman GE, Moffitt PD, Bala GA, Webb RH (1999) Sulfide removal in reservoir brine by indigenous bacteria. SPE Prod Facil 14:219– 225.

- Johnson RJ, Folwell BD, Wirekoh A, Frenzel M, Skovhus TL (2017). Reservoir souring–latest developments for application and mitigation. J Biotechnol 256:57–67.

- Kaster, K. M., et al. "Souring: A Critical Review." Journal of Petroleum Technology, vol. 66, no. 3, 2014, pp. 28–38, https://doi:10.2118/0314-0028-JPT.

- Kennes, Christian. (2018). Bioconversion Processes. Fermentation. 4. 21. 10.3390/fermentation4020021. https://doi:10.3390/fermentation4020021.

- Khan, MAA, Hussain M and. Djavanrood F (2021). Microbiologically influenced corrosion in oil and gas industries: A review. Int. J. Corros. Scale Inhib.,10(1), 80–106. https://doi:10.17675/2305-6894-2021-10-1-5.

- Koshlaf E, Ball AS. Soil bioremediation approaches for petroleum hydrocarbon polluted environments. AIMS Microbiol. 2017 Jan 19;3(1):25-49. PMID: 31294147; PMCID: PMC6604977. https://doi:10.3934/microbiol.2017.1.25.

- Labena A, Husien S, Elhady R, Chapter 13 - Biological souring and mitigation strategies in oil reservoirs, Editor(s): Sanket Joshi, Prashant Jadhawar, Asheesh Kumar, Challenges and Recent Advances in Sustainable Oil and Gas Recovery and Transportation, Gulf Professional Publishing, 2023, Pages 283-301, ISBN 9780323993043, https://doi.org/10.1016/B978-0-323-99304-3.00009-1.

- Lei Zhou, Jie Gao, Yu-Xuan Li, Jun Wu, Biao Wang, Yi-Fan Liu, Shi-Zhong Yang, Ji-Dong Gu, Bo-Zhong Mu, A new control strategy of sulfate-reducing microorganisms and corrosion rate from a high-temperature oil reservoir by nitrate/nitrite coupling with nitrate-reducing bacteria, International Biodeterioration & Biodegradation, 201, 2025, 106065, ISSN 0964-8305, https://doi.org/10.1016/j.ibiod.2025.16065.

- Ma, C.; Xu, X.; Yang, J.; Cheng, L. (2023). Safety Monitoring and Management of Reservoir and Dams. Water 2023, 15, 1078. https://doi.org/10.3390/w15061078.

- Ma, Xiaoyu & Wang, Jida & Gharari, S. & Mizukami, Naoki & Lettenmaier, Dennis. (2025).  Monitoring reservoir storage using SWOT satellite observations and reservoir operation models. Rhttps://10.22541/essoar.175095716.62626354/v1.

- Mahajan S, Tyagi A, Prajapat G, Agrawal A. Advances in production related petroleum microbiology. Adv Appl Microbiol. 2025;131:65-90. Epub 2025 Jun 24. PMID: 40818841 https://doi:10.1016/bs.aambs.2025.04.001.

- Marietou A (2016) Nitrate reduction in sulfate-reducing bacteria. FEMS Microbiol Lett 363:fnw155.

- Nabawy, Bassem. (2025). New approaches in reservoir characterization utilizing conventional and special core analyses: a comprehensive review. Journal of Umm Al-Qura University for Applied Sciences. 1-32. https://10.1007/s43994-025-00225-6.

- Odeyemi, Olushola & Alaba, Peter. (2024). Microbiologically Influenced Corrosion in Oil Fields: Mechanisms, Detection, and Mitigation Strategies. https://doi:10.5772/intechopen.1005181.

- Okoro, C., Ekeng, E., Ohwo, S. et al. (2026). Anaerobic biodegradation and activities of methanogens, acetogens, sulphate and nitrate reducing bacteria in aged oil wells from low sulphate oil producing environment. Biodegradation 37, 14. https://doi.org/10.1007/s10532-025-10224-3.

- Okpala, Gloria & Voordouw, Gerrit. (2018). Comparison of Nitrate and Perchlorate in Controlling Sulfidogenesis in Heavy Oil-Containing Bioreactors. Frontiers in Microbiology. 9. 2423. https://doi:10.3389/fmicb.2018.02423.

- Pannekens M, Kroll L, Müller H, Mbow FT, Meckenstock RU (2019) Oilreservoirs, an exceptional habitat for microorganisms. NewBiotechnol 49:1–9.

- Paulraj, Balachandar & Bhatt, Sachin & Shivarudra, Ashwini & Kavuri, Swethasri & Mehra, Ankur. (2025).  Innovative Strategies for Optimizing Operational Efficiency in Tech-Driven Organizations.

- Priya Saxena, Dipayan Samanta, Etienne Z. Gnimpieba, Kian Mau Goh, Rajesh K. Sani (2025). Sulfate-reducing bacteria: Unraveling biofilm complexity, stress adaptation, and strategies for corrosion control, Science of The Total Environment, 998, 180226,

- Qiao, L., Wang, J., Wei, S. et al. The soil microbial carbon pump for carbon sequestration. Environ Chem Lett 23, 1145–1151 (2025). https://doi.org/10.1007/s10311-025-01861-4.

- Rajput, S., Pathak, R.K. (2025). Reservoir Engineering Excellence: A Geoscientist’s Blueprint. In: Seismic Exploration to Reservoir Excellence. Springer, Singapore. https://doi.org/10.1007/978-981-96-1293-2_9.

- Ramy, Charbel & Ripeanu, Razvan George & Nassreddine, Salim & Tanase, Maria & Zouein, Elias & Diniță, Alin & Muresan, Constantin. (2025). Recent Advances in Stimulation Techniques for Unconventional Oil Reservoir and Simulation of Fluid Dynamics Using Predictive Model of Flow Production. Processes. 13. 1-41. https://10.3390/pr13041138.

- Saradva, Harshil. "Technology Focus: Reservoir Simulation (July 2025)." J Pet Technol 77 (2025): 1–2. doi: https://doi.org/10.2118/0725-0008-JPT.

- Satter A, Iqbal GM. (2016). 8 - Petroleum reservoir management processes, Editor(s): Abdus Satter, Ghulam M. Iqbal, Reservoir Engineering, Gulf Professional Publishing, Pages 137-153, ISBN 9780128002193, https://doi.org/10.1016/B978-0-12-800219-3.00008-5.

- Shah BA, Muthalif AG (2025), "A comprehensive review on corrosion management in oil and gas pipeline: methods and technologies for corrosion prevention, inspection and monitoring". Anti-Corrosion Methods and Materials, Vol. 72 No. 5 pp. 681–701, https://doi.org/10.1108/ACMM-09-2024-3085.

- Shao, S., Zhou, F., Wang, Y., & Wu, J.  (2025). An Integrated Multi-Sensor Information System for Real-Time Reservoir Monitoring and Management. Sensors, 25(18), 5730. https://doi.org/10.3390/s25185730.

- Shib Sankar Ganguli, Vijay Prasad Dimri, Chapter One - Reservoir characterization: State-of-the-art, key challenges and ways forward, Editor(s): Shib Sankar Ganguli, Vijay Prasad Dimri, Developments in Structural Geology and Tectonics, Elsevier, Volume 6, 2023, Pages 1-35, ISSN 2542-9000, ISBN 9780323995931, https://doi.org/10.1016/B978-0-323-99593-1.

- Sindi, Mohammed & Zhu, Xiangyang & Sherry, Angela & Gray, Neil & Christgen, Beate & Head, Ian. (2025). Effects of Produced Water Re-Injection (PWRI) on Reservoir Souring and Microbial Community Dynamics. https://10.1201/9781003469643-12.

- Szymczak, A., Ryo, M., Roy, J., & Rillig, M. C. (2020). Diversity of Growth Responses of Soil Saprobic Fungi to Recurring Heat Events. Frontiers in Microbiology, 11, 521701. https://doi.org/10.3389/fmicb.2020.01326.

- Telang AJ, Ebert S, Foght JM, Westlake D, Jenneman GE, Gevertz D, Voordouw G (1997) Effect of nitrate injection on the microbial community in an oil field as monitored by reverse sample genome probing. Appl Environ Microbiol 63:1785–1793.

- Thakur GC (1996). What Is Reservoir Management? . J Pet Technol 48 (06): 520–525. Paper Number: SPE-26289-JPT. https://doi.org/10.2118/26289-JPT.

- Trutschel, L. R., Chadwick, G. L., Kruger, B., Blank, J. G., Brazelton, W. J., Dart, E. R., & Rowe, A. R. (2022). Investigation of microbial metabolisms in an extremely high pH marine-like terrestrial serpentinizing system: Ney Springs. Science of The Total Environment, 836, 155492. https://doi.org/10.1016/j.scitotenv.2022.155492.

- Vance I, Thrasher, DR. (2005). Reservoir Souring: Mechanisms and Prevention. 123-142. https://doi.org/10.1128/9781555817589.ch7.

- Vance, I., & Thrasher, D. R. "Reservoir Souring: Mechanisms and Prevention." Journal of Petroleum Science and Engineering, vol. 55, no. 1-2, 2007, pp. 115–126, https://doi:10.1016/j.petrol.2006.04.014.

- Wang L, Nie Y, Chen X, Xu J, Ji Z, Song W, Wei X, Song X, Wu X-L. Biodegradation of Crude Oil by Nitrate-Reducing, Sulfate-Reducing, and Methanogenic Microbial Communities under High-Pressure Conditions. Microorganisms. 2024; 12(8):1543. https://doi.org/10.3390/microorganisms12081543.

- Wu, H., Cui, H., Fu, C., Li, R., Qi, F., Liu, Z., Yang, G., Xiao, K., & Qiao, M. (2024). Unveiling the crucial role of soil microorganisms in carbon cycling: A review. Science of The Total Environment, 909, 168627. https://doi.org/10.1016/j.scitotenv.2023.168627.

- Zhang F, She YH, Chai LJ, Banat IM, Zhang XT, Shu FC, Wang ZL, Yu LJ, Hou DJ. (2012). Microbial diversity in long-term water-flooded oil reservoirs with different in situ temperatures in China. Sci Rep. 760.. Epub 2012 Oct 23. PMID: 23094135; PMCID: PMC3478584. https://doi:10.1038/srep00760.

- Zhou, L., Gao, J., Li, Y., Wu, J., Wang, B., Liu, Y., Yang, S., Gu, J., & Mu, B. (2025). A new control strategy of sulfate-reducing microorganisms and corrosion rate from a high-temperature oil reservoir by nitrate/nitrite coupling with nitrate-reducing bacteria. International Biodeterioration & Biodegradation, 201, 106065. https://doi.org/10.1016/j.ibiod.2025.106065.