ESTIMATION OF EXCESS LIFETIME CANCER RISK (ELCR) FROM NATURALLY OCCURING RADIOACTIVE MATERIALS IN MILL TAILING SAMPLES OF MAIGANGA MINING SITE, GOMBE STATE, NIGERIA

.

ABSTRACT

Aims: Mining activities have shown to give rise to so much waste (tailings) which may have poisonous elements and lethal to man and the entire ecosystem. Consequently, it affects his health negatively during and after production. This study estimated the excess lifetime cancer risk (ELCR) due to naturally occurring radioactive materials in mill tailing samples of Maiganga mining site, North-Eastern Nigeria. Study design: This study was designed to assess the presence of naturally occurring radioactive materials (NORMs) and to estimate the excess lifetime cancer risk (ELCR) in mill tailing samples of Maiganga mining site. Place and Duration of Study: Maiganga mining site, North-Eastern Nigeria between November 2024 and October 2025 Methodology: A total of ten (10) samples were systematically collected within the mine sites. Using energy dispersive x-ray fluorescence (ED-XRF) spectroscopic technique, the activity concentrations of some selected radionuclides (226Ra, 232Th and 40K) were determined. Results: The findings revealed that, the activity concentrations of 40K range from 182 .79 Bqkg-1 to 389.37 Bqkg-1 with 218 Bqkg-1 as the mean value while 226Ra and 232Th were below the limit of detection. The analysis of radiation risk parameters (Radium equivalent, absorbed dose, annual effective dose equivalent, annual gonadal dose equivalent, external and internal hazard indices and excess lifetime cancer risk) were found to be: 141.86 Bqkg-1, 65.86 nGyh-1, 0.08mSvy-1, 446.55 µSvy-1, 0.38, 0.52 and 0.28×10-3 respectively. Conclusion: The study concludes that Maiganga mining site is radiologically safe for both workers and the general public. Finally, the study recommends continuous radiological screening of the mining site to keep the potential radiation hazards as low as reasonably achievable (ALARA).

Keywords: NORMs, X-ray fluorescence, ELCR, mill tailing, mining sites

1. INTRODUCTION

Coal mining generates substantial quantities of waste that are often toxic and harmful to aquatic life, vegetation, and human health, with adverse effects persisting both during and after extraction (Reference [1]). Mining activities contribute significantly to environmental degradation, ecosystem disruption, and contamination of surrounding areas. During these operations, considerable amounts of topsoil are removed, resulting in the production of large volumes of mine tailings, which often exhibit elevated levels of radioactivity.

These tailings are frequently disposed of on the surface near mining sites, where they are continuously subjected to physical, chemical and biological processes that can mobilise and concentrate naturally occurring radionuclides in the environment. This accumulation can lead to increased radiation exposure to nearby populations (Reference [2]). The impact of mining on the biophysical environment is considerable: while some effects can be quantitatively assessed, others are difficult to estimate. Many published studies emphasise the socio-economic benefits of mining, such as employment generation and business development, often overlooking its severe environmental consequences (Reference [3]).

Naturally occurring radionuclides, such as those from the decay series of uranium-238 (²³⁸U) and thorium-232 (²³²Th), radioactive potassium-40 (⁴⁰K), as well as cosmic radiation, have existed in the Earth since its formation and are widely distributed throughout the crust and atmosphere. In undisturbed natural environments, these radionuclides and their decay products are generally considered radiologically stable (Reference [4]). Their mobilisation through mining activities, however, can significantly alter this stability, posing environmental and public health risks.

In the study area, open surface mining was utilized. This involves large production of mine tailings (waste) gathered around the mine. The dispositions of the tailings are associated with the discharge of radioactive materials into the environment which gradually gets accumulated [4]. Consequently, coal workers and the environ are eventually exposed externally to the radiations from the easily accessible tailings used essentially for farming or residential purposes; and may significantly amount to additional exposure dangers [5]. Therefore, it is paramount to measure the natural radioactivity and assess the radiological hazard indices of mill tailing samples of the study area.  Extensive coal exploration is ongoing at the study area and elevated levels of radiations may lead to cancer risk and numerous health complications, hence the need to constantly assess the mine.

The findings of this study are expected to contribute significantly to the existing body of knowledge on natural radioactivity in Nigeria, providing valuable reference data for future radiological investigations and supporting adherence to safety protocols in both mining and agricultural operations. The generated data will assist regulatory authorities in developing a comprehensive radiological framework aimed at safeguarding human health and ensuring environmental protection. Furthermore, the outcomes of this study will inform policymakers and relevant regulatory agencies in the formulation of sustainable mitigation strategies for managing environmental contamination and conserving ecosystems.

Specifically, the study quantified the concentrations of selected radionuclides—²²⁶Ra, ²³²Th, and ⁴⁰K—in mill tailings samples collected from within the Maiganga coal mine. It also evaluated the absorbed dose rate to assess immediate radiation exposure risks, and calculated the annual effective dose equivalent to estimate potential long-term hazards. Additionally, the study determined external and internal hazard indices to evaluate possible health risks, radium equivalent activity to represent the overall radiological burden, and the excess lifetime cancer risk, providing a comprehensive assessment of both acute and chronic radiological threats associated with mine tailings.

2. material and methods

The study was carried out over a period of two-years in Maiganga mining site, North-Eastern Nigeria between November 2024 and October 2025. Maiganga, a local community in Akko local government area of Gombe state, North-Eastern Nigeria, is situated between latitude 100 02' to 100 05' to longitude 110 06 to 110 08' with a land area of about 20129.47 Acres (48.16 km2) as shown in Figure 1. It shares boarder to the south with Billiri and to the west, with Kumo town, situated on longitude 9°59'19.65"N and 9°59'3.03"N, latitude 11° 8'31.29"E and 11° 9'44.63” [3]. Maiganga lies within Gombe sandstone, adjacent to and above the Pindiga formation. The Maiganga mining activities started in October, 2007 and are carried out by Ashaka Cement, a company owned by a France-based, Lafarge [6].

Figure 1. Map of the study area.

2.1 Collection and laboratory analysis of samples

The materials used include: Mill tailing samples, ED – XRF, Global Positioning System (GPS) hand trowel, polythene bags with zip lock, containers, pen and papers and weighing balance. The samples of mill tailings were carefully collected with the aid of a hand trowel from different points which satisfactorily represented the entire samples dumped around the mining site. The coordinates of each sampling point were recorded. The top layer of mill tailing was scraped off at each sampling location to remove any possibility of contamination from anthropogenic activities.  About 200 g samples were collected at a depth of about 15-20 cm with the aid of a plastic hand trowel to hinder the introduction of any major, minor, or trace elements from a metallic trowel into the field samples.

The sampling technique used for the research was the random method; in order to improve the statistical sensitivity of the samples. The mining site was intentionally (purposefully) stratified into different or multiple sections to achieve even-sampling in order to obtain representative samples across the locations.

The mill tailings samples collected from the Maiganga mining site were initially prepared by open-air drying for a period of seven (7) days at a temperature of 25 °C to remove any residual moisture. The stony tailings were subsequently crushed into fine powder using a mortar and pestle, and sieved through a wire mesh with 0.5 mm openings to obtain uniform particle sizes. The processed samples were then packaged in appropriately labelled high-density polythene bags to facilitate identification and to contain any potential radioactive gases. Approximately 200 g of each sample was sealed in the bags and stored for a period of 30 days prior to analysis, allowing sufficient time for secular equilibrium to be established between ²²⁶Ra, ²³²Th, and their short-lived decay products (offspring) before transport to the laboratory for measurement [4,7].

Quantitative elemental analysis of the samples was performed using an Energy Dispersive X-ray Fluorescence (ED-XRF) spectrometer (Rigaku NEX DE) equipped with a fifteen-place sample changer and spin function operating in a slow and steady rotation mode. Measurements were conducted under a helium (He) atmosphere using a palladium (Pd) X-ray tube operated at 60 kV and 10 µA, with a 10 mm beam spot size. The detector system comprised a silicon (Si) drift detector with a Peltier cooling circuit. Standardless calibration was applied through the Fundamental Parameters (FP) method using Rigaku RPF-SQX software, enabling the quantification of elements in samples with unknown composition. The software automatically estimated elemental concentrations and applied appropriate corrections. Concentrations of the radionuclides were determined across a range from low parts-per-million (ppm) to percentage weight, and these data were subsequently employed to calculate radiation doses and associated hazard indices.

2.1.1 Radiation Parameters and Hazard Indices

In order to assess the radiation exposure to workers and public in the Maiganga coal mining site some radiation parameters are put up to estimate possible exposure and inferred risks associated. The followings are the parameters considered

2.1.1.1 Absorbed Dose Rate (D)

The quantified levels of activity for 226Ra, 232Th, and 40K were transformed into doses that made use of the factors of transformation 0.462, 0.604, and 0.0417 respectively. These coefficients were employed in the computation of the overall dose rate D, (nGyh-1) as presented in equation (1) below.

D= 0.429ARa +0.666ATh+0.042AK    (nGyh-1)                                        	(1)

where ARa ,ATh and AK are the radionuclide concentration (𝐵𝑞/k𝑔) for 226Ra, 232Th, and 40K in soil and mill tailings samples [8].

2.1.1.2 Annual Effective Dose Equivalent (AEDE):

The AEDE was evaluated using the conversion factors of 0.70 SvGy-1, which transform absorbed dose rate in the air to effective dose and the outdoor occupancy factor of 0.2, with the presumption that, an individual has spent mean of 80% of his time indoor [5][8].  UNSCEAR reports suggested outdoor occupancy factor of 0.2 and conversion coefficient from absorbed radiation in air to effective dose of 0.7 Sv/Gy received by humans must be put into cognizance when estimating the annual effective dose rates [6]. As such AEDE was calculated using equation 2 [8].

AEDE = DR × 8760h × 0.7S × 0.2 × 10-6(Sv/Gy)                                 	(2)

2.1.1.3 Annual Gonadal Dose Equivalent (AGDE)

The AGDE calculates the impact of 226Ra, 232Th, and 40K activities on organs like gonads, bone marrow and bone cells (in other words, it evaluates the dose received by reproductive organs such as gonads, bones marrow and bone cells). Equation 3, as presented by Shams et al. [9] was used to calculate AGDE using conversion factors for the respective isotopes.

AGDE (µSv/y) =3.09ARa  + 4.18ATh +0.314AK                                          	(3)

2.1.1.4 Radium Equivalent Activity (Raeq):

This serves as a commonly used risk indicator. UNSCEAR [8] defined radium equivalent (Raeq) mathematically as:

Raeq(Bqkg)=ARa  +1.43ATh  +0.077Ak                                                                	(4)

Where ARa,ATh and Ak  indicates the activity concentrations of 226Ra, 232Th and 40K respectively. The assumption applies specifically to external hazard caused by gamma rays in building materials. For safe used, less than  370Bqkg-1is recommended as the maximum Raeq value.

2.1.1.5 External Hazard Indices:

Two indices (Hin and Hex) are aimed at restricting radiation exposure to a limit of unity (1) equivalent. These two indices are indicators of external and internal hazards. The external hazard index (Hex ) is obtained from the same expression of Raeq with the assumption that, its maximum value corresponds to the upper limit of  370 Bqkg-1 . It represents the hazard incurred as a result of external exposure from 226Ra, 232Th and 40K in mill tailing samples as shown in equation (5) [8].

Hex = ARa370 (BqKg-1) +ATh259(Bq kg-1) +AK4810 (BqKg-1 ) ≤1                                           	(5)

Where ARa, ATh and AK are the activity concentrations of 226Ra, 232Th and 40K [4][8][10].

2.1.1.6 Excess Lifetime Cancer Risk (ELCR):

Excess Lifetime Cancer Risk (ELCR) is the likelihood of cancer development due to radiation exposure over a lifetime [4][11]. Equation (8) as presented by Taskin et al [11] was used to calculate the excess lifetime cancer risk (ELCR) taking into cognizance the annual effective dose equivalent, life expectancy (LE) or duration of life (DL) and risk factor (RF). For children, its 50 years and 70 years for adults, and the low dose background radiation risk factor of 5% (0.05Sv-1) for public exposure which was believed to cause stochastic effect.

ELCR = AEDE × LE(DL) × RF                          					(6)

3. results and discussion

3.1 Results

3.1.1 Discussion of results

The results of activity concentrations in parts-per million (ppm) for radium and thorium and percentage weight (%wt.) for potassium obtained from mill tailing samples of Maiganga mining site are presented on table 1 in which 226Ra and 232Th were below the limit of detection while 40K has 0.584 %wt. and 1.244 %wt. respectively as the minimum and maximum values with 0.845 %wt. as the mean.

Table 1: Activity Concentrations of Radionuclides (ppm and wt.%) in tailing samples from Maiganga mining site

| S/N | Sample I.D. | Coordinates | 226Ra (ppm) | 232Th (ppm) | 40K (wt.) |
| --- | --- | --- | --- | --- | --- |
| 1 | MMS 1 | 90 98’62’’N, 110 15’18’’E | BLD | BLD | 1.244 |
| 2 | MMS 2 | 90 98’60’’N, 110 15’16’’E | BLD | BLD | 0.874 |
| 3 | MMS 3 | 90 98’63’’N, 110 15’17’’E | BLD | BLD | 0.824 |
| 4 | MMS 4 | 90 98’65’’N, 110 15’15’’E | BLD | BLD | 0.754 |
| 5 | MMS 5 | 90 97’88’’N, 110 16’20’’E | BLD | BLD | 0.584 |
| 6 | MMS 6 | 90 96’98’’N, 110 16’18’’E | BLD | BLD | 1.124 |
| 7 | MMS 7 | 90 96’80’’N, 110 17’22’’E | BLD | BLD | 0.614 |
| 8 | MMS 8 | 90 98’58’’N, 110 15’16’’E | BLD | BLD | 0.864 |
| 9 | MMS 9 | 90 97’67’’N, 110 16’21’’E | BLD | BLD | 0.164 |
| 10 | MMS 10 | 90 95’79’’N, 110 15’19’’E | BLD | BLD | 0.954 |
|  |  | Minimum |  |  | 0.584 |
|  |  | Maximum |  |  | 1.244 |
|  |  | Mean |  |  | 0.845 |

* BLD =Below Limit of Detection, PPM = Parts-per million and %wt. = Percentage weight.

Table 2: Activity Concentrations of the Radionuclides (Bqkg-1) in tailing samples from Maiganga mining site

| S/N | Sample I.D. | Coordinates | 226Ra (Bqkg-1) | 232Th (Bqkg-1) | 40K (Bqkg-1) |
| --- | --- | --- | --- | --- | --- |
| 1 | MMS 1 | 90 98’62’’N, 110 15’18’’E | BLD | BLD | 389.37 |
| 2 | MMS 2 | 90 98’60’’N, 110 15’16’’E | BLD | BLD | 273.56 |
| 3 | MMS 3 | 90 98’63’’N, 110 15’17’’E | BLD | BLD | 257.91 |
| 4 | MMS 4 | 90 98’65’’N, 110 15’15’’E | BLD | BLD | 236.01 |
| 5 | MMS 5 | 90 97’88’’N, 110 16’20’’E | BLD | BLD | 182.79 |
| 6 | MMS 6 | 90 96’98’’N, 110 16’18’’E | BLD | BLD | 351.82 |
| 7 | MMS 7 | 90 96’80’’N, 110 17’22’’E | BLD | BLD | 192.18 |
| 8 | MMS 8 | 90 98’58’’N, 110 15’16’’E | BLD | BLD | 370.43 |
| 9 | MMS 9 | 90 97’67’’N, 110 16’21’’E | BLD | BLD | 192.18 |
| 10 | MMS 10 | 90 95’79’’N, 110 15’19’’E | BLD | BLD | 298.6 |
|  |  | Mean |  | - | 218.22 |
|  |  | Minimum |  | - | 182.79 |
|  |  | Maximum | - | - | 389.37 |
|  |  | Global Standard | 35 | 30 | 400 |

Table 3: Radiation Doses (Raeq, D, AEDE and AGDE) in tailing samples from Maiganga mining site

| S/N | Sample I.D. | Raeq (Bqkg-1) | D (nGyh-1) | AEDE (mSvy-1) | AGDE (µSvy-1) |
| --- | --- | --- | --- | --- | --- |
| 1 | MMS 1 | 151.48 | 71.10 | 0.0872 | 485.76 |
| 2 | MMS 2 | 142.56 | 66.24 | 0.0812 | 449.40 |
| 3 | MMS 3 | 141.36 | 65.58 | 0.0804 | 444.48 |
| 4 | MMS 4 | 139.67 | 64.66 | 0.0793 | 437.61 |
| 5 | MMS 5 | 135.57 | 62.43 | 0.0766 | 420.90 |
| 6 | MMS 6 | 148.59 | 69.53 | 0.0853 | 473.97 |
| 7 | MMS 7 | 136.30 | 62.82 | 0.0770 | 423.84 |
| 8 | MMS 8 | 142.32 | 66.11 | 0.0811 | 448.42 |
| 9 | MMS 9 | 136.30 | 62.82 | 0.0770 | 423.84 |
| 10 | MMS 10 | 144.49 | 67.30 | 0.0825 | 457.26 |
|  | Mean | 141.86 | 65.86 | 0.0808 | 446.55 |
|  | Minimum | 136.30 | 62.43 | 0.0766 | 420.90 |
|  | Maximum | 151.48 | 71.10 | 0.0872 | 485.76 |
|  | Global Standard | 370.00 | 84.00 | 1.0000 | 300.00 |

*Default LLDs applied per NNRA 2023 S4.3.2(c) for conservative risk assessment.

Table 3 is for the radiation doses (Raeq, D, AEDE and AGDE) for the mill tailing samples of the study area. The respective values obtained were: 151.48 to 136.30, 62.43 to 71.10, 0.0766 to 0.0872 and 420.90 to 485.76; and 141.86 Bqkg-1, 65.86 nGyh-1, 0.0808 mSvy-1 and 446.55 µSvy-1 as their respective means. All values were within the safety limit except AEDE with a mean value of 446.55 µSvy-1.

Table 4: Radiation Hazard Indices (Hext and ELCR) from Tailing samples of Maiganga site

| S/N | Sample I.D. | Hex | ELCR (x 10-3) |
| --- | --- | --- | --- |
| 1 | MMS 1 | 0.0810 | 0.3052 |
| 2 | MMS 2 | 0.0569 | 0.2843 |
| 3 | MMS 3 | 0.0536 | 0.2815 |
| 4 | MMS 4 | 0.0491 | 0.2775 |
| 5 | MMS 5 | 0.0380 | 0.2680 |
| 6 | MMS 6 | 0.0731 | 0.2985 |
| 7 | MMS 7 | 0.0400 | 0.2696 |
| 8 | MMS 8 | 0.0770 | 0.2838 |
| 9 | MMS 9 | 0.0400 | 0.2696 |
| 10 | MMS 10 | 0.0621 | 0.2889 |
|  | Mean | 0.0571 | 0.2827 |
|  | Minimum | 0.0380 | 0.2680 |
|  | Maximum | 0.0810 | 0.3052 |
|  | Global Standard | 1.0000 | 0.2900 |

*Default LLDs applied per NNRA 2023 S4.3.2(c) for conservative risk assessment.

While table 4 present the radiation hazard indices (Hext  and ELCR). Minimum and maximum values range from 0.0380 to 0.0810 and 0.2680 to 0.3052. Their mean values were 0.0571 and 0.2827.

3.1 Discussion of Activity Concentrations

The activity concentrations from mill tailings samples of the study area in parts per million and percentage weight; and Bqkg-1 were presented in tables 1 and 2 respectively.

The results of activity concentrations in parts-per million (ppm) for radium and thorium and percentage weight (%wt.) for potassium obtained from mill tailing samples of Maiganga mining site are presented on table 1 (226Ra and 232Th were below limit of detection while 40K has 0.584 %wt. and 1.244 %wt. respectively as the minimum and maximum values with 0.845 %wt. as the mean). While in table 2 above, the values of the activity concentrations of the mill tailing samples of Maiganga mining site in Bqkg-1 were obtained. Both radium and thorium were below the limit of detection (BDL) while potassium values range from 182.79 to 389.37 with 218 Bqkg-1 as the mean value. All the values obtained did not exceed the safety limit of 400 Bqkg-1 set as the global standard. The values of potassium in sample were between 182.79 to 389.37 with 218 Bqkg-1 as the mean. These values of potassium obtained did not exceed the reference levels established by UNSCEAR [8]. There was no wide disparity between the results at the various sampling points and this may be due to similarity in the geological and topographical formation of the study area which have been influenced by mining activities. The results obtained were higher than the findings of Khandaker et al. [5] who recorded a mean value of 70.44 Bqkg-1 but lower than the findings of Davou and Mangset [13], Laniyan and Adewumi [15], Manyoba et al [16] whose values were found as 5901 Bqkg-1, 570.65 Bqkg-1, & 2013.97±35.87 Bqkg-1 respectively as the mean value of potassium, and which are far above the safety limit of 400 Bqkg-1set by UNSCEAR [8]. Radium and thorium concentrations in the samples were below the instrument minimum limit of detection which were similar to the reports of Omale et al [17] and Isaac et al.,[18].

3.1.1 Discussion of Radiation Doses (Raeq, D, AEDE and AGDE)

Table 3 presented the calculated radiation doses of 141.86 Bqkg-1, 65.86 nGyh-1, 0.08 mSvy-1 and 446.55 µSvy-1 as the mean of Raeq, D, AEDE and AGDE respectively. AGDE exceeded the global safety limit of 300 µSvy-1. High values indicate potential genetic and reproductive health risk. Khandaker et al. [5] recorded 42.67 Bqkg-1, 19.14 nGyh-1, 0.02 mSvy-1 and 132.97 µSvy-1 respectively (values were within the safety limit) for the radiation dose parameters. However, Davou and Mangset [14] found higher values that were far above the safety limit. These values were: 25896.16 Bqkg-1 and 86250.95mSvy-1 and 13877mSvy-1 and 76348.49 µSvy-1 (outdoor and indoor) respectively for the dose parameters. Similarly, Manyoba et al [16] obtained 890 Bqkg-1 and 403.54 nGyh-1 as the value of Raeq and D respectively as against 370 Bqkg-1 and 84 nGyh-1 set out by UNSCEAR as the safety limit.

3.1.1.1 Discussion of Radiation Hazard Indices (Hext and ELCR)

In table 4 radiation hazard indices (Hext and ELCR) respectively were presented. The mean values were 0.0571 and 0.2827×10-3 respectively. All the results presented in the table 4 above, were within the safety limit. These findings are in agreement with the results obtained by Khandaker et al. [5] who found 0.02 and 0.02×10-3 respectively as Hext and ELCR but completely differs with the results of Davou and Mangset [14] (2015) who obtained the following results: 25896.16 Bqkg-1, 86250.95 μSvyr-1(outdoor) and 13877.58 μSvyr-1, 76348.49 Svyr-, 70.20 Svyr-1, 71.98 Svyr-1 and 48571.52×10-3. Moreso, Manyoba et al [16] recorded 4.1 as the external hazard indices while Mwinanzi et al [19] obtained 0.39 ×10-3 as excess lifetime cancer risk in a similar study as against 0.29 ×10-3 which was set out by UNSCEAR as the safety limit for excess lifetime cancer risk.

4. Conclusion

Excess lifetime cancer risk (ELCR) from naturally radionuclides in mill tailing samples of Maiganga mining site was estimated using Energy Dispersive X-ray Fluorescence (ED-XRF) technique. The activity concentrations of the measured radionuclides and the radiation doses (D, AEDE, AGDE,  Raeq,  Hext and  ELCR) with the exception of AGDE, were below the safety limit set by UNSCEAR for both human and environmental protection. Furthermore, the results have proven that, there is no likelihood of cancer development. Hence, the study recommends continuous radiological screening of the mining site annually to keep the potential radiation hazards as low as reasonably achievable (ALARA). Coal waste (tailings) should be stored in lined ponds to prevent leaching of radionuclides into groundwater and strict compliance measures should be enforced to all the mining industries so as to keep the environment radiologically safe.

Disclaimer (Artificial intelligence)

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

References

- Amosu, C.O. (2021). Implication of Mining to Health in Maiganga Coal Mine, Gombe State, 	Nigeria. Indian Journal of Management and Language (IJML). 1 (2):1-14.

- Usikalu, M. R., Ajibola, O., Adagunodo T. A. & Akinpelu, A. (2018). Radioactivity Concentration 	and Dose Assessment of soil Samples in Cement Factory and Environs in Ogun State, Nigeria. International Journal of Civil Engineering and Technology. 9(9):10471059

- Abubakar, G. M., Nzotta, C. C., Chiegwu, U. H., Ugwu, A.C., Akosile, C.O., Nwodo K. V. & 	Ezenma, I.C. (2022). Radiological Hazards Assessment in Maiganga Coal 	Mine and Environs, Gombe State North-East Nigeria. Nigerian Journal of Medical Imaging and Radiation Therapy 11 (1):64-71.

- Rabiu, J. A. & Nafiu, M. (2024). Assessment of Natural Radioactivity and Radiation Hazard in Soil and Rock Samples from mining sites within Nort-Eastern 	Nigeria. Unpublished M.Sc. Dissertation, Submitted to the Department Physics, Federal University of Kashere, Gombe, Nigeria.

- Khandaker, M.U., Abdullah, W.H.B. Amin, Y.M.  & Kolo, M.T. (2017). Radionuclide concentrations and excess lifetime cancer risk due to gamma radioactivity in tailing enriched soil around Maiganga coal mine, Northeast Nigeria. International Journal of Radiation Research. 15 (1):72-80.

- Malik, S. (2023). Coal: The Course of a Resource, International Centre for Investigative Reporting, accessed February, 2023.

- Ibrahim, S. & El-Yakub, A. I. (2024). Estimation of Excess Lifetime Cancer Risk and Radiation Hazard Indices in Yunusari, Yobe State, Nigeria. Fane-Fane Int’l Multidisciplinary Journal, 8(3):148-161.

- UNSCEAR (2000). Sources and Effect of Ionizing Radiation. Report To General Assembly, With Scientific Annexes (7 & 11).

- Shams, A., Issa, M. & Alaseri, S.M. (2015). Determination of Natural Radioactivity and Associated Radiological Risk in Building Materials used in Tabuk area, Saudi Arabia. Int. J. Adv. Tech. Vol. 82, pp. 45-62.

- Orgun, Y., Altmsoy, N., Shin, S. Y., Gungor, Y., Gultekin, A. H., Karahan, G. & Karacik, Z. (2007). Natural and Anthropogenic Radionuclides in Rocks and Beach Sands 	from Ezine Region (Canakkale), Western Anatolia, Turkey. Apll Radiat Isot., Vol. 65, pp739-47.

- Ravinsaka, R., Chandramohan, J., Chandrasekaran, A., Prince P. J. J., Vijayagopal, P. & Venkatraman, B. (2015). Assessment of radioactivity concentration of natural Radionuclides and Radiological Hazard Indices in Sediments Samples from the East Coast of Tamilnadu, India with Statistical Analysis. Mar Pollut. Bull., 97(1-2): 419-430.

- Taskin, H., Karavus, M., Ay, P., Topuzoglu, A., Hindiroglu, S. & Karhan, G. (2009). Radionuclides 	Concentrations in Soil and Lifetime Cancer Risk Due to Gamma Radioactivity in Kirklareli, Turkey. J. Env. Rad., Vol.100, PP. 49-53.

- Nigerian Nuclear Regulatory Authority Guidelines on Mining Residues (2023). Specifically Annex 11 for mill tailings management.

- Davou, L.C. and Mangset, W.E. (2015). Evaluation of Radiation Hazard Indices and Excess Lifetime Cancer Risk Due to Natural Radioactivity in Mined Tailings in Some Locations in JOS Plateau State Nigeria. IOSR Journal of Applied Physics (IOSR-JAP) 7(1):67-72 DOI: 10.9790/4861-07116772 e-ISSN: 2278-4861.

- Laniyan, T. A. & Adewumi, A. J. (2021). Health Risk Profile of Natural Radionuclides in Soils, Sediments, Tailings and Rocks aound Mining Sites in Nigeria. Environ Earth Sci, 80, 373. https://doi.org/10.1007/s12665-021-09674-8.

- Manyoba, N., Olukotun, S. F., John, S. O. O. & Mathuthu, M. (2025). Health Impact Assessment of Naturally Occuring Radioactive Materials (NORMs) from a Gold Mine Tailings in Mpumalangs Province, South Africa. Polytechnica, Pp.1-9. https://doi.org/10.1007/s41050-025-00055-9

- Omale, P.E., Okeniyi S.O., Faruruwa, M.D. & Ngokat, A.B. (2014). Determination For Levels of Radionuclides of Uranium, Thorium and Potassium in Water, Sediments and Algae Samples from Selected Coastal Areas of Lagos, Nigeria; Using Energy Dispersive X-Ray Flourescence. Global Journal of Pure and Applied 	Chemistry Research Vol.2, No.1, pp.1-24,

- Isaac E., Maitera O. N., Donatus R. B., Riki Y. E., Yerima E. A., Tadzabia K. & Joseph B. (2019). Energy dispersive X-Ray fluorescence determination of minor and major elements in soils of Mambilla Plateau Northeastern Nigeria. Journal of Environmental Chemistry and Ecotoxicology. Vol. 11(2), pp. 22-28. DOI:10.5897/JECE2018.0442

- Mwinanzi, J. M., Haneklaus, N. H., Bituh, T., Brink, H., Kugiu, K., Lolila, F., Marwa, J. J., Rwiza, M. J. & Mtei, K. M. (2025). Radioactivity Distribution in Soil, Rocks and Tailings at the Geita Gold Mine in Tanzania. Journal of Radiation Research and Applied Sciences. (8): 1-13.