| Journal Name: | Current Journal of Applied Science and Technology |
| --- | --- |
| Manuscript Number: | Ms_CJAST_152210 |
| Title of the Manuscript: | Analysis of Water Mass Flow Rate Influence on a Crystalline Silicon Cell Temperature and Electronic Parameters under Concentrated Light |
| Type of the Article | Research Article |

| ## PART 1: Comments |  |  |
| --- | --- | --- |
|  | ## Reviewer’s comment | Author’s Feedback (It is mandatory that authors should write his/her feedback here) ## We don’t use artificial intelligence or assisted review comments during peer review. |
| Please write a few sentences regarding the importance of this manuscript for the scientific community. A minimum of 3-4 sentences may be required for this part. | This manuscript provides a detailed quantitative analysis of the interaction between cooling fluid dynamics and the fundamental electronic properties of silicon solar cells under concentrated light. By developing and solving both thermal and electrical models, the study determines how the water mass flow rate impacts intrinsic parameters—such as carrier mobility, diffusion coefficients, and intrinsic density—alongside the resultant cell temperature. These findings offer valuable insights for optimizing the design and operational strategies of Concentrated Photovoltaic-Thermal (CPVT) hybrid systems, particularly by predicting the cooling effect limitations and the subtle electronic consequences on parameters like the open-circuit voltage and short-circuit current. |  |
| Is the title of the article suitable? (If not please suggest an alternative title) | YES |  |
| ## Is the abstract of the article comprehensive? Do you suggest the addition (or deletion) of some points in this section? Please write your suggestions here. | No it is comprehensive |  |
| ## Is the manuscript scientifically, correct? Please write here. | The manuscript is found to be scientifically correct as it follows rigorous modelling and methodology with good consistency in results with sufficient novelty and focus. |  |
| Are the references sufficient and recent? If you have suggestions of additional references, please mention them in the review form. | The references are sufficient and recent. |  |
| ## Is the language/English quality of the article suitable for scholarly communications? | The English language needs to be improved throughout the text. |  |
| ## Optional/General comments | The study specifically aims to address a gap in previous research by investigating intrinsic photovoltaic parameters (like carrier mobility and intrinsic carrier density) and how they are affected by the cooling mechanism, which adds scientific value. The authors are requested to carefully go through these questions and provide satisfactory explanations wherever necessary: - The thermal model assumes a uniform PV cell temperature due to its thinness. Given that the cell is subjected to concentrated light and cooled by water flowing in a specific channel, could the authors elaborate on the validation or justification for this uniformity assumption, especially at high concentrations? - The study incorporates an electric field term due to the electron concentration gradient in Equation 16 and mentions this is relevant due to the high level of illumination. Could the authors quantitatively justify why this term becomes significant above 50 Suns and describe how neglecting it would have affected the final carrier density and predicted electrical parameters? - The manuscript states that the electron's lifetime (τ) is supposed to be constant. Since the device temperature varies significantly and carrier lifetime in silicon is typically temperature-dependent, how does this assumption affect the accuracy of the resulting carrier density and subsequent electrical parameter predictions? - The conclusion notes that the effect of mass flow on intrinsic density is sufficient to predict slight variations in open-circuit voltage. Could the authors specifically link the observed change in carrier density near the junction (Figure 10) to the expected change in Voc​ using fundamental solar cell equations. - Equation 2 defines the electric power using the photo conversion efficiency (η) which is temperature-dependent (Equation 3). Could the authors explain how the value of η is determined/modeled at each concentration and mass flow rate, especially since η itself depends on output voltage and current, which are the parameters being influenced by the thermal effects? - Equations 8 to 15 show that intrinsic silicon parameters are strongly temperature-dependent. What is the primary physical mechanism linking temperature rise (due to concentrated light) to the decrease in carrier mobility and diffusion coefficients (Figures 4-7)? Is it primarily phonon scattering or increased impurity scattering? - The n-type doping level is Nd​=1018 cm−3 and the p-type is Na​=1016 cm−3. How do these specific doping levels influence the temperature dependence of the mobility coefficients as calculated by Equations 8-12, particularly concerning the influence of scattering mechanisms? The study concludes that the effect of mass flow on carrier density is insufficient to cause significant variations in current at intermediate and short-circuit conditions. Since increased mass flow leads to a cooler cell and better intrinsic properties (higher mobility/diffusion), which should theoretically increase the generated current, could the authors provide a more detailed analysis of why these improvements are masked or canceled out such that the junction current remains practically invariant? | - The heat balance is established under steady-state conditions. So the PV cell temperature is that of the steady state. This assumes uniformity of temperature. The thinness of the PV cell quickly allows this steady-state to be reached. - Due to the illumination high level, there is a strong generation of electrons near the junction. This generation of electrons decreases with the cell base depth x. An electric potential difference therefore appears in the PV cell base. This electric potential difference in turn causes an electric field to appear and contributes to the conduction of charge carriers. - Our system is hybrid with cooling, and the output temperatures range from 294.10K to 342.24K. In this temperature range, the electrons lifetime can be assumed to be constant. - Photo-voltage is given by the Boltzmann approximation and depends on the carrier density at the junction. In open-circuit situation, the photo voltage corresponds to Vco and also depends on the carrier density at the junction. It is therefore clear that variations in carrier density at the junction cause similar variations in open-circuit voltage Vco. - The electrical efficiency of a hybrid system (CPVT) is the ratio of the maximum electrical power output to the concentrated light power received by the PV cell: - Is it primarily phonon scattering - In this study, Nd and Na have no influence on the cell temperature. These are doping levels of the emitter and the base in our electrical model. - Cooling mainly affects the cell temperature. However, it is known that the photovoltage is more sensitive to temperature variations. The photocurrent, on the other hand, remains practically insensitive to temperature variations. As a continuation of this work, we conducted another study on the influence of mass flow rate on electrical parameters. But we did not wish to discuss this study here. |

| PART 2: |  |  |
| --- | --- | --- |
|  | ## Reviewer’s comment | Author’s Feedback (It is mandatory that authors should write his/her feedback here) |
| Are there ethical issues in this manuscript? | (If yes, Kindly please write down the ethical issues here in detail) No ethical issue was found | No, there are no ethical issues in this manuscript |