Original Research Article

Effects of Flood Events on Socio-Economic Activities and Groundwater Quality in Dschang, West Cameroon

.

ABSTRACT

Aims: To assess the effects of flood events on socio-economic activities and the water quality of wells and boreholes in the Dschang community, West Region, Cameroon. Study design: Cross-sectional descriptive study combining household surveys and water quality analysis.. Place and Duration of Study: Dschang community, West Region, Cameroon. The study was conducted during the post-flood period, following recent flood events in the area. Methodology: A total of 150 households, including male and female respondents, were surveyed using a structured questionnaire. The questionnaire covered demographic characteristics and flood impacts on occupation, education, health, housing, water supply, and sanitation. Water samples were collected from wells and boreholes after flooding and analyzed for physicochemical parameters including pH, electrical conductivity, and total dissolved solids, as well as microbiological indicators including fecal coliforms and streptococci. Results: Out of 150 households surveyed, riverbed overflow was identified as the leading cause of flooding by 28.7 percent of respondents. Flooding negatively affected livelihoods, housing conditions, and access to safe water. Malaria was reported as the most common flood-related disease, affecting 60 percent of households. Water quality analysis showed pH values ranging from 5.6 to 7.5, indicating slight acidification in some sources. Conductivity and total dissolved solids varied across sampling points. Fecal coliforms and streptococci were detected in several wells and boreholes, indicating microbial contamination after flood events. Conclusion: Flooding in Dschang significantly affected socio-economic well-being and compromised the microbiological quality of groundwater sources. Strengthening health service capacity, establishing community-based disaster management committees, and investing in flood risk reduction, water safety measures, infrastructure improvement, and community capacity building are required to reduce future flood impacts.

Keywords: Socio-economic activities, Floods, Impact, Water quality, Cameroon

1. INTRODUCTION

Floods stand out as the most frequent, damaging, and deadly natural disasters worldwide (Zorn, 2018). Over the past 50 years, they have accounted for nearly half of all deaths from natural disasters and approximately one-third of global economic losses (Lu et al. 2022; Sovacool et al. 2015). The devastating impact of floods extends beyond immediate destruction, posing a grave threat to social and economic infrastructures and hindering the attainment of Sustainable Development Goals (SDGs), including poverty reduction (SDG 1), eradication of hunger (SDG 2), ensuring good health and well-being (SDG 3), and universal access to clean water and sanitation (SDG 6) (Leal-Filho et al. 2023; Dube, 2020; Zorn, 2018). Both human activities and climate change have contributed to the increased likelihood and severity of flooding incidents (Cianconi & Janir, 2020; Zscheischler et al. 2018; Kundzewicz etal., 2014; Pall et al. 2011).

In the global context, flooding remains the primary natural disaster, claiming around 20,000 lives annually (Cirella et al. 2019). Urban communities, ill-equipped to combat these extreme weather events, bear the brunt of their devastating consequences (Nicola, 2016; Adetokunbo & Emeka, 2015). Rapid urbanization and population growth exacerbate the challenges, making flood management more complex and costly (Miller & Hutchins, 2017).

In Cameroon, the lack of effective flood risk management in cities has intensified the impact, making flood occurrences increasingly prevalent, notably in cities such as Douala, Yaoundé, Garoua, and Maroua (Bang, 2022; Djomekui & Yemmafouo, 2022; Yengoh et al. 2017). This situation further extends to the catchment areas of various study zones, resulting in significant socio-economic, health, and psychological impacts on vulnerable groups. The disruption of markets due to floods leads to food insecurity, conflicts, and climate-induced migrations (Akanwa et al. 2023; Atanga & Tankpa, 2021), adversely affecting the well-being of the population. A critical peak in flooding was witnessed in the commune of Dschang in August 2022, during which vulnerable populations experienced severe distress. The deluged roads, impassable riverbeds, and destroyed bridges compounded the suffering of affected communities. The destruction of crops in certain areas added to the plight of victims, leading some to abandon their homes in search of better accommodations elsewhere.

To mitigate flood risks and improve living conditions for the local population, the Commune of Dschang has undertaken a large-scale drainage project covering an area of 561 230 m2. This project involves the eviction of people residing on public land and the demolition of associated structures. To inform decision-making processes, especially in vulnerable localities, an upstream study is indispensable to document the consequences of these floods on the population and the environment. The present research represents a significant step in this direction, aiming to assess the socio-economic, environmental, and health impact of flooding on the quality of life and living standards of households in several flood-prone areas of the Commune of Dschang (Régie, Madagascar, and Ngui) village.

2. material and methods

  - Description of the study area

The study was conducted in the Commune of Dschang, located in the Department of Menoua, West Cameroon Region (Fig. 1). The Commune was established by Decree no. 2007/117 on 24 April 2007, and it encompasses approximately 47 urban neighborhoods and 99 rural villages. The population of the Commune is cosmopolitan, with the dominant Bamiléké ethnic group alongside the Mob, Haoussa, Bamouns, Bassa, and Ewondo ethnic groups. The Commune covers a total physical surface area of 252 km², divided into five groups: Foto (99 km²), Foréké-Dschang (86 km²), Fongo-Ndeng (31 km²), Fossong Wentcheng (18 km²), Fotetsa (11 km²), and the Urban center (7 km²). The study area experiences an equatorial monsoon climate with a mountain facies determined by its average altitude of 1400m. This climate is characterized by a rainy season from mid-March to mid-November, followed by a dry season from mid-November to mid-March. While the annual average rainfall is 1,872.3mm, occasional disturbances due to global climate change have been observed, leading to variations in rainfall amounts. The area's altitude influences temperature, with the warmest month (March) averaging 21°2, and the coolest month (July or August) ranging from 19°5 to 18°9. The average annual temperature range is relatively small at around 3°C. The study area maintains high relative humidity throughout the year, with an annual average of 83%. Sunshine duration ranges from 8.5 hours per day in the dry season to 2.2 hours per day in the rainy season.

Dschang is characterized by a mountainous relief featuring hills, valleys, and steep slopes with flooded valleys. The agro-ecological zones in the area comprise low, medium, and high altitudes, offering significant agricultural potential. The soil in the Commune of Dschang consists mainly of brown earth derived from basic rocks, and specific features include syntectonic granites, anatexites, old, and young basalts. Hydromorphic soils develop in marshy lowlands, while ferralitic and hydromorphic soils are influenced by human activities such as agriculture and construction. The Commune is crisscrossed by numerous watercourses, forming a dense hydrographic network. Some rivers, like the Anzin de Fongo-Ndeng, may dry up during the dry season, while others, such as the Ntsée legoh, are prone to regular flooding during the rainy season. Fish species like tilapia, catfish, and carp are abundant in various rivers, providing a vital source of livelihood for local inhabitants. Regarding vegetation, the Commune falls within the montane forest zone, but centuries of human activities have significantly transformed the vegetation cover. Shrubby savannah and raffia palm now dominate the landscape, and anthropogenic vegetation, including Eucalyptus and fir trees, can be found in both urban and rural areas. Fruit trees, such as avocados and mangoes, are occasionally present, along with ubiquitous banana plantations.

The population in the Commune of Dschang is accustomed to different forms of mobility, including students traveling to and from the town when classes are in session, periodic movements of people to coffee plantations on the Mbos plain, and definitive departures of students concluding their studies at the University of Dschang. The area has experienced spatial extension of the urban perimeter, with preferential poles for urbanization in the Ngui district (southwest), Foto hill (northeast), and areas around the Catholic Mission (northwest).

Fig.1. Map location of Dschang

  - Data collection

A combination of secondary and primary data sources was employed to gain a comprehensive understanding of the socio-economic impact of flooding in the study area. Secondary data formed a significant component of this study, incorporating a wide range of existing information. Various sources were meticulously reviewed, including previously conducted research works, newspaper publications, local and international journals, books, maps, and paper presentations from seminars and workshops. These diverse resources provided valuable insights into the historical patterns of flooding, past impacts on communities, and the broader context of flood management in the region.

To augment the secondary data, primary data collection was conducted using a comprehensive questionnaire survey. This approach facilitated direct engagement with the local population, enabling the research team to gather firsthand information on the current socio-economic conditions and the effects of recent flooding incidents. The questionnaire survey was carefully designed to cover key socio-economic indicators, including income levels, livelihood patterns, housing conditions, infrastructure vulnerability, public health concerns, and social cohesion within the selected communities.

  - Sample size and Sampling technique

To collect data from households, 150 households were selected from the 3 main villages subject to flooding events in the community of Dschang. The stratified random sampling technique was used to select samples for the study. The communities form the strata and the sample size of 150 was allocated as follows (Table 1):

Table 1. Sample seize (households)

| s/n | Study areas | Number of households |
| --- | --- | --- |
| 1 | Ngui | 90 |
| 2 | Madagascar | 40 |
| 3 | Régie | 20 |
|  | Total | 150 |

- Validity of instrument: The researchers’ judgment was based on the certification of the instrument by experts that it is measuring what it is supposed to measure. The instrument was given to colleagues and lecturers for critiquing. The resulting critiquing and corrections made by these experts further improved the instruments and increased their content validity.

- Reliability of instrument: A pilot study was conducted where 20 questionnaires were administered and tested for internal consistencies of responses and it had a positive and fluid administration indicating a very strong reliability. Therefore, the instrument is reliable for the study.

  - Water Sampling and Laboratory Analysis

Water Sampling: In order to assess the impact of flooding on the various groundwater resources, 15 water samples (eight wells and seven borehole water samples) were taken and sent to the laboratory for analysis of various physicochemical and microbiological parameters. Table 2 below shows the distribution of the various samples taken in the study areas and according to the different sampling sources. Water samples were collected in 1litre plastic containers and prior to collection as part of our quality control measures all the bottles were washed with non-ionic detergent and rinsed with de-ionized water prior to usage. Before the final water sampling was done, the bottles were rinsed three times with the sample water at the point of collection. Each bottle was labelled according to sampling location while all the samples were preserved at 4ºC and transported to the laboratory.

Analytical methods: All the samples were analyzed for the following physico-chemical parameters and; bacteriological parameters: Temperature (T), Hydrogen Potential (pH), Conductivity (Cnd), Salinity (Sal), Oxidation Reduction Potential (eh), Total Dissolved Solids (TDS), Ammonium (NH4+), Faecal Coliforms (FC), Faecal Streptococci (FS). The physico-chemical and bacteriological analyses of the water samples were carried out according to standard analytical methods (APHA, 1992).

Table 2. Distribution of the various samples taken in the study areas

| Study Areas | Number of Water Samples per Source of Collection |
| --- | --- |
| Ngui | 5 well water samples;4 borehole water samples |
| Madagascar | 2 well water samples; 2 borehole water samples |
| Régie | 1 well water sample; 1 borehole water sample |
| Total | 15 water samples |

  - Data analysis

The data obtained was analyzed through the use of descriptive statistics, such as percentage, frequency table etc.

3. results and discussion

  - Results

    - Socio-demographic distribution of inhabitants of the study area

Table 3 shows that majority of the respondents (62%) are male, while females account for 38%. The largest group is married individuals (44.7%), followed by widowed (19.3%), single (16.0%), and divorced (13.3%) individuals. The table also shows that most of the respondents are married (44.7%), followed by widowed (19.3%), single (16.0%), and divorced (13.3%) individuals. Respondents had diverse occupational backgrounds. Trading (26.0%) and private sector employment (18.7%) are the most common occupations, then farmers (16.7%) and civil servants (12.0%). The majority have at least a primary education (35.3%) and secondary education (31.3%). A smaller proportion (22.7%) has no formal education and some had tertiary education (10.7%). The majority (57.3%) have household sizes of 5-9 members, while smaller households with 1-4 members constituted 41.3%. A significant majority of respondents were landowners (71.3%), while 28.7% were tenants.

Table 3. Socioeconomic activities of the inhabitants of the study area

| Variable | Frequency | Percentage (%) |
| --- | --- | --- |
| Gender |  |  |
| Male | 93 | 62 |
| Female | 57 | 38,0 |
| Marital status |  |  |
| Single | 24 | 16,0 |
| Married | 67 | 44,7 |
| Widowed | 29 | 19,3 |
| Divorced | 20 | 13,3 |
| Occupation |  |  |
| Civil servant | 18 | 12,0 |
| Private sector employee | 28 | 18,7 |
| Farmer | 25 | 16,7 |
| Trading | 39 | 26,0 |
| Unemployment | 23 | 15,3 |
| Retired | 17 | 11,3 |
| Education |  |  |
| No formal education | 34 | 22,7 |
| Primary | 53 | 35,3 |
| Secondary | 47 | 31,3 |
| Tertiary | 16 | 10,7 |
| Household seize |  |  |
| 1-4 | 62 | 41,3 |
| 5-9 | 86 | 57,3 |
| 10 & above | 2 | 1,3 |
| Land ownership status |  |  |
| Tenant | 43 | 28,7 |
| Owner | 107 | 71,3 |

Source: Field survey analysis

    - Nature of houses in the study area

Table 4 reveals that the majority of respondents (98.7%) live in ground floor houses, while a very small proportion (1.3%) reside in split-level houses. The distribution of housing by nature indicates that a significant portion of respondents live in clay houses (41.3%), followed by hard (34.0%) and semi-hard (24.7%) housing types. Most of habitats are located in wetlands (66.0%), which suggest a higher exposure to flooding due to their proximity to water bodies. The remaining 34.0% are situated in non-wetland areas. A large proportion of habitats (78.7%) are within the alluvial zone, which is prone to flooding. Only a smaller percentage (21.3%) is located outside this zone, potentially indicating relatively lower flood risk. Also, more than half of the habitats (56.7%) were categorized as precarious, which could imply a greater susceptibility to flood-related damages and disruptions. A portion of respondents engaged in small-scale animal rearing at home (28.0%). The primary source of drinking water is tap water (46.0%), followed by boreholes (42.7%), springs (9.3%), and a very small percentage use well (2.0%). As for the toilet type, most respondents use traditional toilets (80.7%), while a smaller proportion use manual flush toilets (14.7%). Some respondents (4.7%) use a combination of both toilet types.

Table 4. Description of houses in the study area

| Variable | Frequency | Percentage |
| --- | --- | --- |
| Type of housing |  |  |
| Split-level house | 2 | 1,3 |
| Ground floor house | 148 | 98,7 |
| Nature of housing |  |  |
| Hard | 51 | 34,0 |
| Semi-hard | 37 | 24,7 |
| Clay | 62 | 41,3 |
| Location |  |  |
| Habitat located in wetlands | 99 | 66,0 |
| Habitat located in a non-wetland area | 51 | 34,0 |
| location of habitat in relation to the watercourse |  |  |
| Habitats within the alluvial zone | 118 | 78,7 |
| Habitats outside the alluvial zone | 32 | 21,3 |
| Precarity of housing |  |  |
| Precarious habitats | 85 | 56,7 |
| Non-precarious habitats | 65 | 43,3 |
| At-home animal rearing |  |  |
| small-scale rearing at home | 42 | 28,0 |
| No small-scale rearing at home | 108 | 72,0 |
| Drinking water supply |  |  |
| Tap water | 69 | 46,0 |
| Well | 3 | 2,0 |
| Borehole | 64 | 42,7 |
| Spring | 14 | 9,3 |
| Toilet type |  |  |
| Manual flush toilet | 22 | 14,7 |
| Traditional toilet | 121 | 80,7 |
| Manual flush toilet and traditional toilet | 7 | 4,7 |

Source: Field survey analysis

    - Impact of floods in the study area

      - Awareness and reasons for living in flood prone areas

Fig.2, reveals that majority (94.7 %) of the respondents are aware of living in flood prone areas. The reasons for living in these flood areas in spite this awareness as shown in Fig.3, are Cultural and traditional ties (29.4 %), lack of alternatives (22.5 %), limited housing options (21.2 %), lack of land ownership (16.5 %) and livelihood (10.4 %).

Fig.2. Awareness of flooding in study area

Fig.3. Reasons for living in flood prone areas.

      - Intensity of flooding event

Fig.4 shows that, a substantial portion (46.0%) of respondents reported being totally flooded, while 38.0% of respondents reported experiencing partial flooding. Some respondents (16%) however mentioned that they did not experience any flooding.

Fig.4. Intensity of flooding in the community

      - Reasons for Flooding

The primary cause of flooding, as identified by 28.7% of respondents, is the overflow of the river. A notable 25.3% of respondents attributed flooding to the filling of the watercourse bed with waste. About 22.0% of respondents cited torrential rains, rainstorms, and a lack of proper gutter systems as contributing factors to flooding. Inadequate drainage infrastructure can hinder the efficient flow of water, leading to localized flooding. Around 14.0% of respondents attributed flooding to construction in swampy areas. A smaller proportion (10.0%) of respondents mentioned a combination of torrential rains and the filling of the streambed with waste as causes of flooding (Fig.5).

Fig.5. Causes of flooding in the community

      - Impact of flooding on household properties

According to Fig.6, the most frequently reported type of property loss is household appliances (31.1%), about 25.0% of respondents reported the loss of food stock due to flooding. Flooding also led to clothing losses for 18.2% of respondents. Approximately 14.2% of respondents reported the loss of seed stock. Interestingly, 11.5% of respondents reported not experiencing any property losses. This could be attributed to various factors, including flood preparedness measures, the location of their residences, or luck in avoiding severe flooding incidents.

Fig.6. Loss of household properties

    - Water quality after flooding event

The physicochemical and bacteriological characteristics of water from the investigated boreholes in Ngui, Madascagar, and the Régie were assessed after the flooding event (Table 5). The temperature of the sampled wells ranged from 24.6 ºC to 26.4 ºC, with the WHO guideline at 25 ºC. The pH values varied between 5.54 (Regie well) and 7.12 (Ngui well 3), which was within the WHO guideline range (6.5–8.5). Conductivity values were between 184.1 µS/cm (Madascagar well 1) and 860 µS/cm (Ngui well 2), compared to the WHO guideline of 400 µS/cm. Salinity ranged from 0.09 ppt (Madascagar well 1) to 0.43 ppt (Ngui well 2). Redox potential values were generally negative, ranging from –74 mV (Ngui well 3) to 14 mV (Regie well). Total Dissolved Solids (TDS) varied between 129 ppm (Madascagar well 1) and 605 ppm (Ngui well 2), with the WHO reference at 267 ppm. Ammonium concentrations were in the range of 0.50 mg/l (Madascagar well 1) to 1.02 mg/l (Ngui well 4), compared with the WHO value of 0.5 mg/l. Faecal coliform counts ranged from 10 CFU/100 ml (Madascagar well 1) to 215 CFU/100 ml (Ngui well 4), the WHO guideline recommends a 0 CFU/100 ml. Faecal streptococci were detected between 3 CFU/100 ml (Madascagar well 1) and 85 CFU/100 ml (Ngui well 4), also against a WHO guideline of 0 CFU/100 ml.

The physicochemical and bacteriological characteristics of water from the investigated boreholes were also assessed after the flooding event (Table 6). Temperatures ranged from 24.2 °C in the Régie borehole to 26.3 °C in Ngui borehole 2, with an average close to the WHO guideline value of 25 °C. The pH values varied between 5.60 in Ngui borehole 2 and 7.51 in Madascagar borehole 2, while conductivity showed values from 117.3 µS/cm (Régie borehole) to 381 µS/cm (Madascagar borehole 2). Salinity levels were low, ranging between 0.05 ppt and 0.19 ppt across all boreholes. Redox potential values fluctuated considerably, from a minimum of −80 mV at Madascagar borehole 2 to a maximum of +33 mV at Ngui borehole 2. Total dissolved solids concentrations ranged between 78.4 ppm in the Régie borehole and 265 ppm in Madascagar borehole 2. Ammonium levels were detected between 0.10 mg/L (Régie borehole) and 0.50 mg/L (Ngui borehole 3). Regarding microbiological indicators, faecal coliforms were observed in four boreholes, with values ranging from 1 to 3 CFU/100 ml, while faecal streptococci were present in three boreholes with concentrations between 1 and 2 CFU/100 ml with the WHO guideline recommending 0 CFU/100ml).

Table 5. Water quality of wells in study site after flooding event.

| Parameters | Units | Ngui Well 1 | Ngui well 2 | Ngui Well 3 | Ngui well 4 | Ngui well 5 | Madascagar well 1 | Madascagar well 2 | Regie well | WHO Drinking water guideline |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature (T) | ºC | 26.4 | 24.8 | 25.6 | 25.6 | 25.7 | 24.6 | 24.6 | 25 | 25 |
| Hydrogen potential (pH) | / | 6.96 | 7.01 | 7.12 | 7.07 | 6.04 | 6.11 | 5.54 | 7 | 6.5-8.5 |
| Conductivity (Cnd) | µS/Cm | 379 | 860 | 544 | 285 | 184.1 | 415 | 409 | 375 | 400 |
| Salinity (Sal) | ppt | 0.19 | 0.43 | 0.27 | 0.14 | 0.09 | 0.2 | 0.18 | 0.19 | / |
| Redox potential (eh) | mV | -64 | -67 | -74 | -54 | 3 | -1 | 14 | -16 | / |
| Total Dissolved Solids (TDS) | ppm | 266 | 605 | 383 | 383 | 129 | 291 | 267 | 267 | / |
| Ammonium (NH4+) | mg/l | 0.60 | 0.85 | 0.65 | 1.02 | 0.5 | 0.67 | 0.55 | 0.70 | 0.5 |
| Faecal Coliforms (FC) | CFU/100 ml | 87 | 160 | 105 | 215 | 10 | 35 | 34 | 130 | 0 |
| Faecal Streptococci (SF) | CFU/100 ml | 45 | 75 | 32 | 85 | 3 | 17 | 15 | 55 | 0 |

Table 6. Water quality of boreholes in study site after flooding event.

| Parameters | Units | Ngui borehole 1 | Ngui borehole 2 | Ngui borehole 3 | Ngui borehole 4 | Madascagar borehole 1 | Madascagar borehole 2 | borehole Régie | WHO Drinking water guideline |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature (T) | ºC | 24,8 | 26,3 | 26,1 | 25,1 | 26 | 25 | 24,2 | 25 |
| Hydrogen potential (pH) | / | 6,15 | 5,60 | 6,18 | 6,55 | 6,11 | 7,51 | 6,46 | 6,5-8,5 |
| Conductivity (Cnd) | µS/Cm | 131,7 | 211 | 174,9 | 126,4 | 163,4 | 381 | 117,3 | 400 |
| Salinity (Sal) | ppt | 0,06 | 0,10 | 0,09 | 0,06 | 0,08 | 0,19 | 0,05 | / |
| Redox potential (eh) | mV | 1 | 33 | -10 | -17 | 9 | -80 | -23 | / |
| Total Dissolved Solids (TDS) | ppm | 90,3 | 148 | 121 | 86,9 | 114 | 265 | 78,4 | / |
| Ammonium (NH4+) | mg/l | 0,4 | 0,13 | 0,5 | 0,3 | 0,18 | 0,21 | 0,10 | 0,5 |
| Faecal Coliforms (FC) | CFU/100 ml | 1 | 0 | 3 | 1 | 0 | 0 | 0 | 0 |
| Faecal Streptococci (SF) | CFU/100 ml | 1 | 0 | 2 | 1 | 0 | 0 | 0 | 0 |

    - Prevalence of diseases in the study area

The principal disease reported by respondents (Fig.7) after flooding was malaria (60.0 %), followed by diarrheal disease (25.0 %) and typhoid (15.0 %). According to the respondents (Fig.8) these diseases affects mostly children (72.7 %), elderly people (21.8 %) and to a certain level adult (5.6 %). Fig.9 shows that most households (33.7 %) spend $1-50, followed by 26.5 % that report spending $101-150. Some households (15.1 %) find themselves spending more than $200 and 6.0 % spend $151-200.

Fig.7. Various diseases encountered after flooding

Fig.8. Category of affected by diseases after flooding

Fig.9. Amount of money spent by households for health issues

    - Resilient adaptations to flood incidents in the study area

Table 7 shows that, the major copping strategies utilized by individuals in the communities include building defensive barriers and raising house foundation (33.9 %), elevate doorsteps and pit latrines (27.2 %), other strategies include installation of pipe exits (16.8 %) and utilization of sandbags and timber logs (22.1 %). The main adaptation technique employed at community level for flood adaptation is Installation of effective waste disposal methods (60.0 %) and establishment of drainage systems and construction of barriers (37.6 %). The table shows that government’s involvement in flood control has been inadequate as reported by 93.3 % of the respondents.  Most of the respondents agreed that government have undertaken some measures for flood management such as cleansing of drainage catchment by local administrative bodies (54.1 %), public knowledge/dissemination (27.8 %) and enforcement of rules against haphazard waste disposal into drainage systems (18.0 %). In their opinion, absence of community-wide collaboration for flood mitigation (40.6 %), corruption (32.7 %) and lack of awareness regarding efficient flood control methods (26.7 %) were responsible for lack of success in halting the flood menace through community efforts.

Table 7. Measures for resilient adaptation to flood incidents in the study area

| s/n | Items | Frequency | Percentages |
| --- | --- | --- | --- |
| 1 | What coping strategies do individual household members use for flood adaptation ? |  |  |
|  | Building defensive barriers and raising house foundation | 121 | 33.9 |
|  | Elevate doorsteps and pit latrines | 97 | 27.2 |
|  | Installation of pipe exits | 60 | 16.8 |
|  | Utilization of sandbags and timber logs | 79 | 22.1 |
| 2 | What adaptation techniques do the communities employ in response to floods? |  |  |
|  | Installation of effective waste disposal methods | 126 | 60.0 |
|  | Establishment of drainage systems and construction of barriers | 79 | 37.6 |
|  | Regulation of housing development | 5 | 2.4 |
| 3 | How might you evaluate the extent of government engagement in flood management within your locality? |  |  |
|  | Inadequate | 140 | 93.3 |
|  | Moderate | 8 | 5.3 |
|  | Nonexistent/Zero government participation | 0 | 0.0 |
|  | Outstanding | 0 | 0.0 |
|  | Satisfactory | 2 | 1.3 |
| 4 | Have governmental authorities undertaken any measures for flood management using the following approaches within your vicinity? |  |  |
|  | Cleansing of drainage catchment by local administrative bodies | 138 | 54.1 |
|  | Enforcement of rules against haphazard waste disposal into drainage systems | 46 | 18.0 |
|  | Public knowledge/dissemination | 71 | 27.8 |
| 5 | In your view, what factors contribute to the inability to effectively combat flooding despite community initiatives ? |  |  |
|  | Absence of community-wide collaboration for flood mitigation | 123 | 40.6 |
|  | Corruption | 99 | 32.7 |
|  | Lack of awareness regarding efficient flood control methods | 81 | 26.7 |

Source: Field survey analysis

    - Priority needs of households following floods

Fig.10 shows that after flooding events access to health facilities (64.7 %) is the priority need of the communities, followed by access to health facilities and food (14.7 %), access to food, health, and shelter (11.3 %), access to shelter (5.3 %), resumption of activities (2.7 %) and 1.3 % reported having no need.

Fig.10. Priority need of households in the study area.

  - Discussion

Gender distribution is crucial for assessing vulnerability, influenced by gender's role in disaster effects, response, and recovery (Bradshaw & Fordham, 2015). Particularly in Dschang's traditional setting, women are susceptible face specific flood challenges due to societal gender roles. In addition, diverse occupations - trading, private sector, farming, civil service - highlight economic flood risks. So then, trade and private sector roles might encounter supply chain disruptions; farmers may experience crop and livestock losses. Likewise, education's disaster readiness impact is notable (Hoffmann & Muttarak, 2017). For this reason, tertiary-educated individuals can enhance Dschang's resilience by sharing knowledge and participating in decisions. Futhermore, majority landownership signifies stability and attachment in Dschang. However, land ownership brings flood-resilient infrastructure upkeep responsibilities. Again, the prevalence of clay houses implies that a significant portion of the community resides in structures more susceptible to flooding damage. Shrestha et al. (2021) similarly found construction material influenced flood damage extent in flood-prone regions. Additionally, the distribution of wetland habitats indicates higher flood exposure due to their proximity to water bodies. This aligns with Baby et al. (2018) findings that areas near water bodies face greater flood risk from overflow during heavy rainfall. Indeed, the reliance on tap water, boreholes, and springs observed in the study area for drinking reflects diverse water sources in communities. According to Ishaq et al. (2022), diversified sources is important for flood-resilient water supply. Consequently, predominant traditional toilet use over manual flush highlights sanitation vulnerability. Paterson et al. (2018) show inadequate sanitation during flooding worsens health risks, emphasizing sanitation strategy implementation.

Moreover, respondents' heightened awareness of residing in flood-prone areas in Dschang signifies a positive grasp of risks and knowledge. This awareness aligns harmoniously with core principles of disaster risk reduction (DRR), providing a solid foundation for fostering community engagement in flood preparedness and mitigation (Nohrstedt et al. 2022). Yet, while awareness stands crucial, the enduring habitation in flood-prone areas is attributed to cultural and traditional ties, highlighting the intricate relationship between the community of Dschang and their environment. Here, cultural and historical bonds often outweigh perceived risk. Resource constraints further constrain vulnerable populations in Dschang from relocating to safer zones, concouring with Adams (2016) mentioning the concept of "trapped populations," where socio-economic factors impede adaptive actions. Similarly, the data highlights river overflow as a significant flooding cause. Hence, rivers, acting as natural water conduits, can overflow during heavy rainfall, leading to adjacent areas being submerged (Mukunoki et al. 2021). This observation aligns with Yang et al. (2018) research, emphasizing the vulnerability of riverine areas to flooding during intense precipitation events. Additionally, improper waste disposal and obstructed watercourses hinder water flow, resulting in localized inundation. This underscores the importance of effective waste management practices in Dschang's flood risk reduction efforts, as highlighted. Consequently, the most reported property loss pertains to household appliances. This reveals the vulnerability of essential domestic items to flooding in Dschang. Such losses can disrupt daily routines, impacting overall quality of life for affected residents. In Dschang, malaria, a vector-borne disease linked to stagnant water, thrives during and after flooding. The prevalence of malaria aligns with Manyangadze et al. (2019) findings, highlighting heightened vector-borne disease risks in flood-prone regions in Mbire district, Zimbabwe. Regarding demographics, children bear the brunt of these diseases, with their developing immune systems and potential exposure to contaminated environments, faces elevated flood health risks (Hanna & Oliva, 2016). The high expenditure brackets suggest that a substantial number of Dschang households’ shoulder significant financial burdens due to flood-related health impacts.

The post-flood water quality results from wells in Dschang indicate slight acidification (pH 5.6–7.5), conductivity fluctuations, and microbiological contamination, which align with patterns observed in other flood-affected regions. Similar studies in Nigeria and Cameroon have shown that floods increase mineral dissolution and transport of contaminants into groundwater, leading to higher TDS and variable pH (Umar et al. 2023; Mbam et al. 2020). The detection of faecal coliforms and streptococci in some wells highlights the risk of faecal intrusion during flooding, a phenomenon widely reported across developing regions (Wright et al. 2004). These results emphasize the vulnerability of local groundwater to flood-driven contamination and the need for preventive and adaptive water management strategies, consistent with WHO (2017) drinking-water safety guidelines.

In the communities, the primary coping strategies are building defensive barriers and elevating house foundations. This concurs with the research conducted by Bertilsson et al. (2019), stressing structural adjustments' role in enhancing household flood resilience. Also, elevating doorsteps and pit latrines also emerge as significant coping methods. This approach underscores safeguarding essential infrastructure from flood impacts. Likewise, effective waste disposal installation stands out as the main community-level adaptation technique. This is in line with Ourloglou et al. (2020) findings linking proper disposal to reduced flood risk. In fact, government involvement in flood control is deemed inadequate, highlighting a vital gap in community-government collaboration crucial for effective flood management. These results align with Bertilsson et al. (2019) emphasis on multi-level governance's importance for successful disaster risk reduction.

In the aftermath of flooding in Dschang, the paramount importance of health facility access is the foremost community need. This highlights health care's critical role in addressing immediate health threats like injuries, waterborne illnesses, and related issues, all of which resonate with Dschang's flood context. These findings align with Louw et al. (2019) insights, emphasizing the crucial role of accessible healthcare during and after flood events, particularly relevant for Dschang's community well-being. Furthermore, the intricate relationship between health and food security becomes evident, accentuating the multifaceted nature of post-flood challenges in Dschang. Insufficient nutrition and food scarcity can escalate health vulnerabilities (Akanwa et al. 2023; Simane et al. 2016). This holistic view reflects the nuanced dynamics of flood aftermath in Dschang, where addressing health and food security intricately intertwines with broader community resilience.

4. Conclusion

The impacts of flooding are widespread, affecting regions across the globe, whether developed or developing. This research unequivocally demonstrates the substantial socio-economic ramifications of flooding within the community of Dschang. These consequences, includes socio-economic devastation, property loss, environmental degradation, and health challenges, emphasizing the urgency of comprehensive flood management. Hence, developing an integrated flood management for sustainable development offers a promising avenue to mitigate these effects. Such an approach promises to forge a secure, hospitable, and aesthetically pleasing living and working environment in Dschang, ultimately catalyzing enhanced socio-economic well-being for its communities.

Recommendations

Based on the findings of this study, the authors recommend possible solutions that would accommodate structural and non-structural measures to minimizing flood problems observed in the study area for sustainable development. These include;

- Strengthening flood preparedness is crucial for Dschang's communities. Developing and implementing effective early warning systems that can provide residents with timely alerts, enabling them to take proactive measures and minimize potential damages. Collaborative efforts among local authorities, community leaders, and relevant agencies are essential for this purpose.

- Engaging communities in flood management decisions can enhance local ownership and effectiveness. Encouraging the adoption of traditional knowledge and local practices that have historically helped mitigate flooding can be valuable. Establishing community-led disaster management committees can facilitate coordination and swift responses during floods.

- Enhancing health services' capacity to handle flood-related health challenges is imperative. This includes developing contingency plans, stockpiling essential medicines, and training healthcare providers in flood-related health risks. Additionally, promoting proper waste disposal practices can prevent water contamination and minimize health risks.

- Advocating for increased government involvement and funding in flood management initiatives is vital. Authorities should allocate resources for flood risk reduction, infrastructure upgrades, and community capacity building. Collaboration with international organizations and donor agencies can also enhance financial support for flood mitigation projects.

Raising awareness about flood risks and proper preparedness measures can empower residents. Implementing educational programs in schools, organizing workshops, and using media platforms can disseminate valuable information on flood prevention, response, and recovery.

Disclaimer (Artificial intelligence)

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

References

- Adams, H. (2016). Why populations persist: Mobility, place attachment and climate change. Population and Environment. https://doi.org/10.1007/s11111-015-0246-3

- Ilesanmi, A., & Mgbemena, E. (2015). Urbanization, housing, homelessness and climate change adaptation in Lagos, Nigeria: Lessons from Asia. Journal of Design and Built Environment, 15(2). https://doi.org/10.22452/jdbe.vol15no2.3

- Akanwa, A. O., Banerjee, A., Jhariya, M. K., Muoghalu, L. N., Okonkwo, A. U., Ikegbunam, F. I., & Madukasi, E. I. (2023). Climate-induced conflicts between rural farmers and cattle herders: Implications on sustainable agriculture and food security in Nigeria. In M. K. Jhariya, A. Banerjee, R. S. Meena, S. Kumar, & A. Raj (Eds.), Ecorestoration for sustainability (pp. 373–416). John Wiley & Sons Ltd. https://doi.org/10.1002/9781119879806.ch14

- Atanga, R. A., & Tankpa, V. (2021). Climate change, flood disaster risk and food security nexus in Northern Ghana. Frontiers in Sustainable Food Systems, 5, 706721. https://doi.org/10.3389/fsufs.2021.706721

- Baby, S. N., Arrowsmith, C., Liu, G. J., Mitchell, D., Al-Ansari, N., & Abbas, N. (2021). Finding areas at risk from floods in a downpour using the LiDAR-based elevation model. Journal of Civil Engineering and Architecture, 15(1), 1–12. https://doi.org/10.17265/1934-7359/2021.01.001

- Bang, H. N. (2022). A concise appraisal of Cameroon’s hazard risk profile: Multi-hazard inventories, causes, consequences and implications for disaster management. GeoHazards, 3(1), 55–87. https://doi.org/10.3390/geohazards3010004

- Bertilsson, L., Wiklund, K., de Moura Tebaldi, I., Rezende, O. M., Veról, A. P., & Miguez, M. G. (2019). Urban flood resilience – A multi-criteria index to integrate flood resilience into urban planning. Journal of Hydrology, 573, 970–982. https://doi.org/10.1016/j.jhydrol.2018.06.052

- Bradshaw, S., & Fordham, M. (2015). Double disaster: Disaster through a gender lens. In A. E. Collins, S. Jones, B. Manyena, & J. Jayawickrama (Eds.), Hazards, risks, and disasters in society (pp. 233–251). Academic Press. https://doi.org/10.1016/B978-0-12-396451-9.00014-7

- Cianconi, P., Betrò, S., & Janiri, L. (2020). The impact of climate change on mental health: A systematic descriptive review. Frontiers in Psychiatry https://doi.org/10.3389/fpsyt.2020.00074

- Cirella, G. T., Iyalomhe, F. O., & Adekola, P. O. (2019). Determinants of flooding and strategies for mitigation: Two-year case study of Benin City. Geosciences, 9(3), 136. https://doi.org/10.3390/geosciences9030136

- Djomekui, B. L. S., & Yemmafouo, A. (2022). Population resilience to flooding in the urbanised mangrove of Douala, Cameroon. International Journal of Disaster Resilience in the Built Environment, 14(2), 135–153. https://doi.org/10.1108/IJDRBE-12-2021-0153

- Dube, E. (2020). The build-back-better concept as a disaster risk reduction strategy for positive reconstruction and sustainable development in Zimbabwe: A literature study. International Journal of Disaster Risk Reduction. https://doi.org/10.1016/j.ijdrr.2019.101401

- Hanna, R., & Oliva, P. (2016). Implications of climate change for children in developing countries. The Future of Children, 26(1), 115–132. https://doi.org/10.1353/foc.2016.0006 .

- Hoffmann, R., & Muttarak, R. (2017). Learn from the past, prepare for the future: Impacts of education and experience on disaster preparedness in the Philippines and Thailand. World Development, 96, 32–51. https://doi.org/10.1016/j.worlddev.2017.02.016

- Ishaq, S., Nahiduzzaman, K. M., Sultana, S. R., Rana, A., Mohammadiun, S., Yousefi, P., Hewage, K., & Sadiq, R. (2022). Flood-resilient governance in Okanagan valley of British Columbia: current practices and future directives. Environmental Reviews, 31(2), 327–347. https://doi.org/10.1139/er-2022-0006

- Kimani-Murage, E. W., Schofield, L., Wekesah, F., Mohamed, S., Mberu, B., Ettarh, R., Egondi, T., Kyobutungi, C., & Ezeh, A. (2014). Vulnerability to food insecurity in urban slums: Experiences from Nairobi, Kenya. Journal of Urban Health, 91(6), 1098–1113. https://doi.org/10.1007/s11524-014-9894-3

- Kundzewicz, Z. W., Kanae, S., Seneviratne, S. I., Handmer, J., Nicholls, N., Peduzzi, P., Mechler, R., Bouwer, L. M., Arnell, N. W., Mach, K., Muir-Wood, R., Brakenridge, G. R., Kron, W., Benito, G., Honda, Y., Takahashi, K., & Sherstyukov, B. (2014). Flood risk and climate change: Global and regional perspectives. Hydrological Sciences Journal, 59(1), 1–28. https://doi.org/10.1080/02626667.2013.857411

- Leal Filho, W., Trevisan, L. V., Rampasso, I. S., Anholon, R., Dinis, M. A. P., Brandli, L. L., & Mazutti, J. (2023). When the alarm bells ring: Why the UN sustainable development goals may not be achieved by 2030. Journal of Cleaner Production, 407, 137108. https://doi.org/10.1016/j.jclepro.2023.137108

- Olanrewaju, C. C., Chitakira, M., Olanrewaju, O. A., & Louw, E. (2019). Impacts of flood disasters in Nigeria: A critical evaluation of health implications and management. Jàmbá: Journal of Disaster Risk Studies, 11(1), a557. https://doi.org/10.4102/jamba.v11i1.557

- Lu, L. C., Chiu, S. Y., Chiu, Y. H., & Chang, T. H. (2022). Sustainability efficiency of climate change and global disasters based on greenhouse gas emissions from the parallel production sectors: A modified dynamic parallel three-stage network DEA model. Journal of Environmental Management, 317, 115401. https://doi.org/10.1016/j.jenvman.2022.115401

- Manyangadze, T., Mavhura, E., Mudavanhu, C., & Pedzisai, E. (2021). An exploratory analysis of the spatial variation of malaria cases and associated household socio-economic factors in flood-prone areas of Mbire district, Zimbabwe. GeoJournal, 87, 3845-3860. https://doi.org/10.1007/s10708-021-10460-z

- Mbam, B. N., Kamgang, K. B., & Nola, M. (2020). Microbiological quality of water sources in the West region of Cameroon: quantitative detection of total coliforms using Micro Biological Survey method. BMC Public Health, 20, 346. https://doi.org/10.1186/s12889-020-8443-0

- Miller, J. D., & Hutchins, M. (2017). The impacts of urbanisation and climate change on urban flooding and urban water quality: A review of the evidence concerning the United Kingdom. Journal of Hydrology: Regional Studies https://doi.org/10.1016/j.ejrh.2017.06.006

- Mukunoki, T., Suetsugu, D., Sako, K., Murakami, S., Fukubayashi, Y., Ishikura, R., Hino, T., Sugimoto, S., Wakinaka, K., Ito, S., & Koyama, A. (2021). Reconnaissance report on geotechnical damage caused by a localized torrential downpour with emergency warning level in Kyushu, Japan. Soils and Foundations, 61(2), 600-620. https://doi.org/10.1016/j.sandf.2021.01.008

- Banks, N. (2016). Urban livelihoods in an era of climate change: Household adaptations and their limitations in Dhaka, Bangladesh. In M. Roy, S. Cawood, M. Hordijk, & D. Hulme (Eds.), Urban poverty and climate change: Life in the slums of Asia, Africa and Latin America (pp. 113-129). Routledge. https://doi.org/10.4324/9781315716435-7

- Nohrstedt, D., Parker, C. F., von Uexkull, N., Mård, J., Albrecht, F., Petrova, K., Nyberg, L., Göteman, M., Hileman, J., Messori, G., & Di Baldassarre, G. (2022). Disaster risk reduction and the limits of truisms: Improving the knowledge and practice interface. International Journal of Disaster Risk Reduction, 67, 102661. https://doi.org/10.1016/j.ijdrr.2021.102661

- Ourloglou, O., Stefanidis, K., & Dimitriou, E. (2020). Assessing Nature-Based and Classical Engineering Solutions for Flood-Risk Reduction in Urban Streams. Journal of Ecological Engineering, 21(2), 46-56. https://doi.org/10.12911/22998993/116349

- Pall, P., Aina, T., Stone, D. A., Stott, P. A., Nozawa, T., Hilberts, A. G., & Allen, M. R. (2011). Anthropogenic greenhouse gas contribution to flood risk in England and Wales in autumn 2000. Nature, 470(7334), 382-385. https://doi.org/10.1038/nature09762

- Paterson, D. L., Wright, H., & Harris, P. N. A. (2018). Health risks of flood disasters. Clinical Infectious Diseases, 67(9), 1450-1454. https://doi.org/10.1093/cid/ciy227

- Shrestha, B. B., Kawasaki, A., & Zin, W. W. (2021). Development of flood damage assessment method for residential areas considering various house types for Bago Region of Myanmar. International Journal of Disaster Risk Reduction, 66, 102602. https://doi.org/10.1016/j.ijdrr.2021.102602

- Simane, B., Beyene, H., Deressa, W., Kumie, A., Berhane, K., & Samet, J. (2016). Review of climate change and health in Ethiopia: Status and gap analysis. The Ethiopian Journal of Health Development, 30(1), 28-41. https://ejhd.org/index.php/ejhd/article/view/453

- Sovacool, B. K., Kryman, M., & Laine, E. (2015). Profiling technological failure and disaster in the energy sector: A comparative analysis of historical energy accidents. Energy, 90, 2016-2027. https://doi.org/10.1016/j.energy.2015.07.043

- Umar, I. M., Babatunde, A., & Adamu, M. (2023). Geophysical and hydrochemical investigation of groundwater quality in Nguru, Nigeria: impact of flooding. FUDMA Journal of Sciences, 7(2), 178-188. https://doi.org/10.33003/fjs-2023-0702-2061

- World Health Organization. (2017). Guidelines for drinking-water quality: Fourth edition incorporating the first addendum. Geneva, World Health Organization. https://iris.who.int/handle/10665/254637

- Wright, J., Gundry, S., & Conroy, R. (2004). Household drinking water in developing countries: a systematic review of microbiological contamination between source and point-of-use. Tropical Medicine and International Health, 9(1), 106-117. https://doi.org/10.1046/j.1365-3156.2003.01160.x

- Yang, L., Scheffran, J., Qin, H., & You, Q. (2015). Climate-related flood risks and urban responses in the Pearl River Delta, China. Regional Environmental Change, 15(2), 379-391. https://doi.org/10.1007/s10113-014-0651-7

- Yengoh, G. T., Fogwe, Z. N., & Armah, F. A. (2017). Floods in the Douala metropolis, Cameroon: attribution to changes in rainfall characteristics or planning failures?. Journal of Environmental Planning and Management, 60(2), 204-230. https://doi.org/10.1080/09640568.2016.1149048

- Zorn, M. (2018). Natural disasters and less developed countries. In S. Pelc & M. Koderman (Eds.), Nature, Tourism and Ethnicity as Drivers of (De)Marginalization: Insights to Marginality from Perspective of Sustainability and Development (Vol. 3, pp. 59-78). Springer. https://doi.org/10.1007/978-3-319-59002-8_4

- Zscheischler, J., Westra, S., Van Den Hurk, B. J. J. M., Seneviratne, S. I., Ward, P. J., Pitman, A., AghaKouchak, A., Bresch, D. N., Leonard, M., Wahl, T., & Zhang, X. (2018). Future climate risk from compound events. Nature Climate Change, 8(6), 469-477. https://doi.org/10.1038/s41558-018-0156-3