# Review Article

Microbial Bioremediation of Heavy Metals in Soil

.

ABSTRACT

Rapidly rising heavy metal (HM) concentrations pose a severe hazard to humans and ecological system owing to a drastic rise in the anthropogenic activities. Heavy metals lead to pollution that has a major influence on plants. Owing to stress caused by heavy metal contamination, plant survival becomes harder, and their overall health suffers. Metal toxicity rehabilitation of polluted soil can be accomplished by chemical as well as physical methods that are typically time-consuming, costly and unsustainable. Toxic HMs (heavy metals) which exceed specific permitted limits have detrimental effect on density, composition and physiological performances of microbiota, along with soil fertility and dynamics, causing drop in production with negative consequences for the food chain, humans and fauna. Consequently, metal-induced phytotoxicity issues must be addressed immediately and urgently to sustain and maintain microbial physiological activities, soil nutrient pools, and greater production in a rapidly degrading environment. Microbes that are resistant to heavy metal can be used in bioremediation to restore damaged environments. Bioremediation is relatively inexpensive as it makes use of natural biological processes and has widespread public acceptance. Accordingly, this review examines the mechanisms and interactions such as stress response and defence mechanisms that occur among microorganisms against heavy metals.

Keywords: Bioremediation, Heavy metals, Microbial community, Environment

.

1. INTRODUCTION

Across a large range of terrestrial ecosystems around the world, heavy metal (HM) stress has assumed grave proportions. Depending on their biological roles and effects, metals are classified into three groups: a) essential metals possessing biological function b) non-essential metals possessing known biological function and c) non-essential metals having an unknown biological purpose. They may act as: i) structural components of numerous enzymes, regulate osmotic pressure, and stabilize molecules via electrostatic interactions, while being involved in redox reactions (Fe, Cu, Mo, Na, K) (Rathoure et al., 2016) ii) toxic metals (Cd, Sn, Ti, Pb, Hg, Al, Ge, As, Sb, and Se) that are harmful to humans and environment (Orisakwe et al., 2019) iii) non-essential, non-toxic and biologically inert (Rb, Sr, Cs and T) metals (Charzyński et al., 2017). Na, K, Ca, Zn, Cu, Co, Mg, Mn and Fe are poisonous to cells if they exceed allowed limits (Prabhakaran et al., 2016; Zoroddu et al., 2019). Agricultural soil contamination with HM is mostly caused by unprecedented use of chemical pesticides and fertilizers, long-term sewage sludge application, dumping of industrial waste, waste incineration, and vehicle exhaust (Saljnikov et al., 2022). Plants and animals are at great risk due to HMs occurring naturally in soils which is further aggravated owing to geologic and anthropogenic activities (Eyankware and Obasi, 2021). Furthermore, since heavy metal particles can travel great distances in gaseous and particle phases (Izydorczyk et al., 2021), they can rapidly accumulate in water, soil and living systems (Abdel-Meged, 2021)). Although small quantities of HMs are crucial for healthy plant growth (Caracciolo and Terenzi, 2021)), excessive quantities are detrimental to plants along with other food-chain species (Jamla et al., 2021). Zinc, molybdenum, copper, manganese, nickel and cobalt play an important role in biological functions and developmental pathways that rely on these HMs (Shahid et al., 2015). However, when these metal concentrations exceed optimum levels in comparison to other toxic metals like cadmium (Cd), arsenic (As), lead (Pb), and mercury (Hg), these can drastically lower crop output (Amari et al., 2017; Pratush et al., 2018; Rahman and Singh, 2019) . Agronomic soil contamination is also caused by volcanic emissions, metal-containing dusts and the breakdown product of HM enriched rocks. These factors are in addition to the rapidly growing industrialization that includes dependency on mining, smelting, disposing of HM wastes, paint, gasoline, fertilizers, biosolids, and other agriculture related activities (Rizvi et al., 2020). Figure 1 depicts the various natural and anthropogeniuc sources of heavy metals.

Heavy metals are potentially cytotoxic, carcinogenic, and mutagenic, and the vast majorityare harmful to humans even at low doses (Edao, 2017)). According to some studies, excess HM concentrations have adverse effects (Table 1) on the brain, heart, kidneys, bones, and liver (Engwa et al., 2019; Rehman et al., 2021). Plants possess many defense mechanisms against HM stress and toxicity, including systems that limit HM uptake, sequester metal into vacuoles, binding of HMs to phyto chelelatins, regulating the intake of metals by transporters (like metalothioneins; metal binding proteins) and enhancement of antioxidant systems (Shahid et al., 2015; Tiwari and Lata, 2018). Similarly, some microorganisms have well developed metabolic capabilities supported by their molecular machinery (Ma et al., 2016), allowing them to adapt and perform even when exposed to high HM levels (Mishra et al., 2017), implying that these microbes can thrive in a metal polluted environment.

Figure 1: Sources of Heavy metals

Phytoremediation (using growing plants to reduce soil HM concentrations) and the employment of rhizospheric microorganisms are becoming increasingly popular as high-performance options. Rhizospheric microorganisms, in particular, can protect plants from HM stress and facilitate the building up in soil of heavy metals (Enebe and Babalola, 2018)). The current review aims to offer an update on the present state of knowledge about rhizosphere bacteria with respect to HM remediation. Additionally, the paper reviews the role of rhizospheric microorganisms in tackling HM stress in plants. Furthermore, though microbial remediation and  plant tolerance to HMs has been researched for several years, interest in further exploration of plant-microbe-metal connections continues to grow because of their direct effects on enhanced biomass production and as HM tolerance  (Tiwari and Lata, 2018; Hansda and Kumar, 2017).

Table 1:  Toxic effects of  heavy metals

| Heavy Metal | EPA Regulatory Limit (ppm) (EPA, 2009) | Toxic Effects | Reference |
| --- | --- | --- | --- |
| Ag | 0.10 | Causes skin and other body tissues to turn grey or blue grey, breathing problems, lung and throat irritation and stomach pain. | (ATSDR 1990) |
| As | 0.01 | Affects essential cellular processes like oxidative phosphorylation and ATP synthesis. | (Tripathi et al., 2007) |
| Ba | 2.0 | Respiratory failure, cardiac arrhythmias, gastrointestinal dysfunction, twitching of muscles and increased blood pressure. | (Jacobs et al., 2002 ) |
| Cd | 5.0 | Endocrine disruptor, carcinogenic, mutagenic, kidney dysfunction, damage to lungs and bones, hinders regulation of calcium in biological systems. | (Degraeve 1981; Bernard 2008; Kaji M 2012) |
| Cr | 0.1 | Lung cancer, hair loss, birth defects. | (Salem et al., 2000) |
| Cu | 1.3 | Damage kidney and brain, increased levels cause liver cirrhosis and chronic anaemia, stomach and intestine irritation. | (Wuana & Okieimen 2011) |
| Hg | 2.0 | Fatigue, drowsiness, autoimmune diseases, loss of hair, insomnia, memory loss, restlessness, vision disturbance, temper outbursts, tremors, brain damage, kidney failure | (Ainza et al., 2010; Gulati et al., 2010 ) |
| Ni | 0.2 | Skin diseases e.g., itching, nose cancer, sinuses, throat and lungs through constant inhalation, affect fertility and hair loss. | (Das et al., 2008) |
| Pb | 15 | Affects neurological, haematological, gastrointestinal, cardiovascular and renal systems. In children, excessive exposure leads to impaired development, short-term loss of memory, deceased intelligence, learning disabilities and coordination problems | (Salem et al., 2000; Wuana and Okieimen, 2011) |
| Se | 50 | Causes malfunctioning of endocrine function, damage to natural killer cell activity, hepatotoxicity, fatigue, gastrointestinal disturbances, dizziness | (Hess and Schmid, 2002) |

- Heavy metal  signaling and plant tolerance

Research on plant response to HM stress has advanced substantially in the previous few decades. Several genes induced by HM stress have been discovered owing to various omics methodologies viz. transcriptome analysis of Brassica, Arabidopsis, and Lycopersicum pointing to the role played by numerous transcription factors (TFs) under HM stress, such as AP2/ERF, bZIP, DREB and bHLH (Singh et al., 2016). Heavy metal stress signal transduction is the process by which the stress signal (s) is first detected by a variety of proteinaceous ion channels and receptors, then by non-protein messengers such as calcium, cyclic nucleotides, and hydrogen ions. Numerous phosphatases and kinases transmit the stress signals, which in turn cause the expression of several TFs and, ultimately, the synthesis of peptides that detoxify metals (Islam et al., 2015; Kumar and Trivedi, 2016). Different signalling pathways in plants, including calcium-dependent signaling (Shi et al., 2018), mitogen activated kinase protein signalling,  reactive oxygen species (ROS) signalling, and hormone signalling, are triggered by heavy metal(s), which increases the expression of TFs and/or  of genes related to stress response (Kumar and Trivedi, 2016). Similarly, the MAPK (Mitogen activated protein kinases) signaling cascade phosphorylates many TFs, including MYC, DREB, ABRE,  bZIP, MYB, WRKY and NAC which have a strong impact response against  metal stress (Jalmi et al., 2018). Furthermore, several phenols, organic acids, amines, amino acids, tocopherol and glutathione influence HM stress resistance in plants (Yusuf et al., 2012). Some of the plant responses to various stresses are so complicated that their roles are yet unclear.

Moreover, biological remediation is commonly regarded the best technique for removing HM toxicity; being an environmentally acceptable, low-cost, and widely accepted technology (Khalid et al., 2017). The release of exudates from roots mobilizes metals through complexation (Etesami and Maheshwari, 2018), along with supplying nutrients (Mishra et al., 2017) and acting as a source of energy to microbial communities (Keiluweit et al., 2015), supporting plant survival and growth (Hayat et al., 2017). In addition, root exudates contain phytochelatins (PC) and amino acids, that act as binding molecules for HM intracellularly (Fremont et al., 2021). Root exudates emit enzymes and protons (H+), which aid in rhizosphere acidification and electron transport thus, resulting in higher metal bioavailability (Ma et al., 2016). Exudate concentration changes brought about by certain HMs promote development of biomarkers. Mass spectrometry, gas chromatography and metabolomics approaches have been employed to demonstrate the effect of lead accumulation on root exudates of Sedum alfredii herb, 15 chemicals have been discovered that are assumed to be potential biomarkers of lead (Pb) contamination (Luo et al., 2017).

Microbes assist in promoting survival and growth of plant under stress due to heavy metal by digesting waste and transforming complicated waste into non-toxic by-products owing to multiple resistance mechanisms that enable them to cope up with harmful HMs (Parmar et al., 2022; Chaudhary et al., 2022). Plant-associated microbes use a variety of remediation techniques, including metal sorption, bioaccumulation, enzymatic oxidation/reduction reactions to convert HMs to non-toxic form, and their efflux from cells (Zhang et al., 2020; Hemdan et al., 2022).

- Heavy metal tolerance by Microbes

Microorganisms have a vital role to play in biogeochemical cycling of HMs by metal reclamation (Ali et al., 2019b; Sharma et al., 2021). Consequently, microbial presence aids in lowering soil HM concentration through a range of mechanisms as passive or fast metal uptake via cell surface adsorption by extracellular component such as polysaccharide and active or slow metal uptake through sequestration involving metallothioneins (MT), which are metal binding proteins. However, because HM toxicity induces poor plant growth, HM bioavailability can stymie the phytoremediation even further (Scragg, 2005)). Plant-associated microbes use a variety of remediation techniques, including metal sorption, Rhizospheres are home to HM-tolerant bacteria that support plant development. These bacteria simultaneously handle these two primary problems by regulating plant growth and altering the physico-chemical characteristics of the soil to raise metal bioavailability, which leads to quick HM detoxification (Khanna et al., 2019; Aparicio et al., 2021). They also decrease soil metal availability by precipitation, acidification, complexation, redox processes and chelation (Etesami, 2018).  HM-PGP (plant growth promoting) bacteria produce organic acids that reduce pH of soil and sequester soluble metal ions (Ali et al., 2019a; Mishra et al., 2021; Parihar et al., 2021). Further, acidic pH levels in rhizosphere promote HM adsorption and bioavailability (Mishra et al., 2017). Fungus filamentous hyphae have been observed to penetrate deeper soil aggregates, leading to HM chelation and adsorption (Mishra et al., 2021; Dada, 2019).

- Heavy metal bioremediation by bacterial species

Firmicutes, Actinobacteria and Proteobacteria, dominate the populations of bacteria in HM polluted locations, with Bacillus, Pseudomonas, and Arthrobacter species dominating (Pires et al., 2012). Bacterial cells can bioremediate heavy metals by a utilizing variety of mechanisms, including: i) adsorption on the cell surface; ii) precipitation outside the cell; iii) intracellular accumulation through the action of specific metal binding proteins called metallothioneins; iv) entrapment and accumulation within extracellular capsules; v) biosorption and bioaccumulation within the cell wall; and vi) intracellular accumulation into vacuoles (Bae et al., 2001; Akkoyun et al., 2020; Açıkel 2011; Ahmad et al., 2016; Hassan et al., 2017; Banik  et al., 2014). Figure 2 depicts various mechanisms used by bacteria for bioremediation of HMs.

Heavy metal remediation in bacteria can also take place by way of synthesis of siderophores facilitated by protein-mediated membrane metal transport and siderophore-metal complex development. Biosurfactants production is another mechanism employed by bacteria for HM remediation. HM remediation caused by biosurfactant takes place either through association of free-form residues of metals with complex ones or accumulation in solid solution interface, leading to metal-biosurfactant interaction (Miller, 1995)). Biosurfactant-metal complex leaves soil surface owing to desorption leading to micelles development (Açıkel 2011). Biosurfactants are produced by a wide range of bacteria, including Acinetobacter sp., Pseudomonas sp., Bacillus sp., and Arthrobacter sp. compared with synthetic substances, biosurfactants have a low or no environmental impact and they can be formed in situ (Sinha & Paul 2012; Zubair et al., 2016). Organic pollutants are mineralized by bacteria into end products viz. water, carbon dioxide or metabolic intermediates that act as primary substrates for growth and development cell in bioremediation process. Microorganisms put up a two-way defense, viz. degradative enzyme production against target pollutants besides resisting heavy metals (Ahmad et al., 2016).

Furthermore, a diverse range of free-living and symbiotic PGPR occurring in soil rhizosphere influence plant development and productivity by generating growth regulators, supplying and assisting in nutrient extraction from soil (Vejan et al., 2016). They limit metal bioavailability by building complexes with siderophores, bacterial transporters and particular metabolites (Rezanka et al., 2019). The HM tolerant PGPRs such as Bacillus, Pseudomonas, Streptomyces, and Methylobacterium boost crop development and production by minimizing the negative effects of HMs (Raklami et al., 2019). Moreover, several rhizobacteria assist metal accumulation, HM uptake and build tolerance of hyper accumulating plants (Thijs et al., 2017). Nitrogen fixation symbiotically has been reported in HM tolerant (HMT) rhizobial strains from polluted sites, despite the fact that nitrogenase enzyme activity and nodulation are prone to HM stress. It is well known that the rhizobia and legume symbiosis detoxifies HM and restores the damaged soils (Checcucci et al., 2017). Despite these practices, HM bioremediation might benefit from using genetically modified bacteria (Rai et al., 2021; Ullah et al., 2021); nevertheless, this remains restricted to laboratory studies (Gupta and Singh, 2017).

The naturally existing microorganisms are either incapable or have limited ability to eliminate chemicals that are hazardous. The capacity of these indigenous bacteria to remove contaminants has been increased by the multidisciplinary use of genetics, bioinformatics, molecular methods, and microbiology; these altered organisms are referred to as genetically modified organisms or genetically engineered microbes (GMOs/GEMs) (Wu et al., 2021). Many naturally or genetically created microbes can breakdown, convert, or chelate many harmful substances, providing better environmental pollution control solutions. Scientists use natural or modified microbes on a regular basis to remove HM toxins, radioactive waste, metalloids, metals and oil products from polluted locations.

The availability of genetic engineering technology provides the possibility of specially tailoring microbial biosorbents with the required selectivity and affinity for heavy metals. Table 2 represents the genetically modified microorganism for bioremediation of heavy metals. Genetically modified bacteria have either one or many genes that promote heavy metal remediation, such as genes related to metal chelators, biodegradative enzyme transporters, metal uptake regulators, metal homeostasis, abiotic and biotic stress tolerance (Singh et al., 2011).  For example, genetically modified bacteria viz. Moraxella sp and E. coli. expresses synthetic phytochelatin EC20 on their cell surface that can collect as much as 25 times more cadmium or mercury as collected by wild-type strains (Bae et al., 2003).

Table 2: Genetically engineered bacteria for HM bioremediation

| S No. | Genetically engineered microbes | Modified Genes | Mode of expression | Reference |
| --- | --- | --- | --- | --- |
| 1 | Ralstonia eutropha | Recombinant rhizobacteria bound on Chambliss | Removes heavy metals | (Sharma, 2020)) |
| 2. | Acidithiobacillus ferrooxidans | Mer C; Ion transporter | Degradation of mercury | (Arshadi and Yaghmaei, 2020)) |
| 3. | Pseudomonas putida X3 strain | Incorporation of EGFP and methyl parathion cadmium degradation gene | Degradation of cadmium and methyl parathion | ( Ayogu et al., 2020) |
| 3 | Rhodopseudomonas palustris | Express mercury transporting system | Degradation of mercury | ( Brown et al., 2020) |
| 4 | E. coli SE5000 strain | nixA; express nickle transporting system | Degradation of nickle from aqueous system | ( Tsyganov et al., 2020) |
| 5 | Mesorhizobium huakuii | Genes from Arabidopsis thaliana encoding PCs | Degradation and accumulation of cadmium | ( Luo et al., 2020) |

However, there is worry that GEMs, when introduced into polluted locations, may cause harmful environmental consequences due to horizontal transfer of recombinant DNA and may compete with native bacterial populations (Dixit et al., 2015; Wu et al., 2006). In other circumstances, the released GEMs do not survive long and perish before having any influence on bioremediation. The ability of GMOs to survive in ecologically stressful environments is crucial to their successful use, and once they have served their intended purpose, there must be a suitable method to remove them from their area of action (Tsyganov et al., 2020).

Figure 2: Heavy metal bioremediation mechanisms employed by bacterial cell

- Heavy metal bioremediation by fungi

Due of their extensive potential for bioremediation applications, fungi are organisms that are receiving more attention. HM stress has been observed to be tolerated by some filamentous fungi from the genera Aspergillus, Penicillium (Khan et al., 2019), Trichoderma, Mucor (Oladipo et al., 2018), Fusarium (Singh et al., 2015), Rhizopus (Srivastava et al., 2011). Various fungi including Fomitopsis meliae, Rhizopus microspores, Trichoderma ghanense and Absidia cylindrospora are able to resist and biosorb iron, lead, arsenic, cadmium and copper (Albert et al., 2018),).  The process of detoxifying heavy metal ions through fungi-metal interactions involves multiple mechanisms viz. i) the metals are mobilized through the production and excretion of fungal products. Such products include citric acid, which is an effective metal-ion chelator, and oxalic acid, which transforms metal ions into insoluble oxalates that contain primary metals resulting from phosphate breakdown. Figure 3 depicts fungal cell mechanisms for heavy metals (Zn, Cu, Ca, Mg, Pb, Ni, Cd, Co, Sr and Mn) bioremediation.

Figure 3: Heavy metal bioremediation mechanisms employed by fungal cell

The organic acids produced by fungi might augment the metal solubility aided by mycosphere acidification and formation of metal-complex structure (Gadd et al., 2014) ii) the chemical behaviour of metals and biomass concentration affect the metal-fungal cell wall interaction; an intricate mechanism comprising several processes e.g., ion exchange, complexation, crystallization, adsorption and precipitation, iii) metal uptake induced by specific metal transporters. The constituents of carriers may include metabolically coupled H+ gradient driven transport system iv) intracellular metal immobilization which comprises of two processes i.e., complexation by cytoplasmic protein (metallothionein and phytochelatins) and vacuoles compartmentation. Metallothioneins are metal binding proteins which modulate intracellular concentrations and bind both essential and inessential metals for example Cu and Cd, respectively. Fungal vacuole plays an important role in molecular degradation, storing of metabolites, regulating the metal ion cytosolic concentrations and detoxification of potentially toxic HM ions. v) Metal transformations are brought about by various kinds of chemical reactions that involve reduction, oxidation, dealkylation and methylation (Srivastava et al., 2011) . These reactions can cause metal volatilization thereby reducing metal toxicity. Metal transportation occurs through cytoplasmic vacuoles and vesicles to plant symbionts and other parts of fungus mycelium.

Species of genera Allescheriella sp. Phlebia sp., Stachybotrys sp., Pleurotus pulmonarius, Botryosphaeria rhodian are remarkable examples of properties of metal binding resulting from the negative charge (-) present on various functional groups viz. carboxylic, amine and phosphate across various components of cell walls of fungi. Arbuscular mycorrhizal fungi are prominent soil microbes that improve surface area of root and nutrient absorption in plants by establishing a direct contact between plant roots and soil ((Saxena et al., 2017). These AMF also reduce host plant’s metal toxicity (Gamalero et al., 2005; Riaz et al., 2021). The filamentous hyphal structures of mycorrhizal and wood-rotting fungi forms oxalate crystals that are reported to halt and detoxify HM by penetrating deep soil aggregates and chelates or adsorbing HM (Gadd et al., 2014). Aspergillus parasitica, Ganoderma carnosum, and Cephalosporium aphidicola biodegrade Pb (II) in toxic soil via the process of biosorption (Akar et al., 2006; Tunali et al., 2006; Akar et al., 2007). In addition to this few Hg resistant fungus have also been reported  (Hymenoscyphus ericae, Neocosmospora vasinfecta, Verticillum terrestreare etc.) that are capable of biotransforming Hg (II) state to ꞵ HgS  and Hg0(Kelly et al., 2006). Thus, detoxifying it.

- Conclusion

Plants contribute to the formation of their own rhizobiome and some plant species may have a unique microbial community that can fluctuate depending on the stage of growth and the location of the plant's roots. Plants can develop a unique rhizobiome that is helpful for their resistance when they are cultivated in polluted soils or under stress situations, like when there are heavy metals present in soil. Microbial communities living in various habitats have developed numerous efficient methods for detoxifying metal-contaminated surroundings. Metal toxicity is mitigated by useful soil microflora using myriads of strategies, including: i) exclusion; ii) active metal transport from cell; iii) intracellular and extracellular requisition; iv) enzyme-mediated degradation of HMs from more toxic to lesser toxic forms; and iv) reduction in cellular metal sensitivity. Soil bacteria and fungi can detoxify a polluted environment by employing single or multiple concurrent methods. Metal clearance procedures may differ depending on microorganism type, metal concentrations and species, and environmental conditions. HM resistant bacteria and fungi have been isolated and traced from several sources e.g., soil, water etc. A microbiological approach to HM tolerance and clean-up is both eco-friendly and cost-effective. However, the commercialization of this technology is still awaited in the absence of ample information. Furthermore, genetically engineered microorganisms adapted to local conditions can effectively cure the soils of HM contamination. Thereby, increase the effectiveness of microbial remediation, however they do have certain moral, legal, and biosafety concerns. Additionally, the effectiveness of GEMs in real-world settings is a constant source of worry. Therefore, in accordance with biosafety regulations, significant research on GEMs is required.  Consequently, the constant loop between research and development to estimate and treat growing concentrations of metal contaminants along with techniques, equipment, and understanding to handle these problems have been this review’s chief concerns.

Disclaimer (Artificial Intelligence)

Author(s)  hereby  declare  that  NO  generative  AI  technologies  such  as  Large  Language  Models  (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

Competing Interests

Authors have declared that no competing interests exist

References

- Abdel Meged, R.M. (2021). Probiotics: a promising generation of heavy metal detoxification. Biol Trace Elem Res. 199:2406–2413. https://doi.org/10.1007/s12011-020-02372-5

- Açıkel, Y.S. (2011). Use of biosurfactants in the removal of heavy metal ions from soils. In: Biomanagement of metal-contaminated soils. Dordrecht: Springer. p. 183–223. https://doi.org/10.1007/978-90-481-9765-3_9

- Agency for Toxic Substances and Disease Registry (ATSDR). (1990). Toxicological profile for silver. Atlanta (GA): U.S. Department of Health and Human Services.

- Ahmad, S., Tabassum, H., Alam, A. (2016). Role of microbial bioremediation of heavy metals from contaminated soils: an update. Int J Biol Pharm Allied Sci. 5:1605–1622.

- Ainza, C., Trevors, J.T., Saier, M.H. (2010). Environmental mercury rising. Water Air Soil Pollut. 205:47–48. https://doi.org/10.1007/s11270-009-0051-y

- Akar, T., Cabuk, A., Tunali, S., Yamac, M. 2006. Biosorption of Pb(II) by Ganoderma carnosum. J Environ Sci Health A. 41:2587–2606. https://doi.org/10.1080/10934520600873536

- Akar, T., Tunali, S., Çabuk, A. (2007). Characterization of Pb(II) biosorption by Aspergillus parasiticus. Appl Biochem Biotechnol. 136:389–405. https://doi.org/10.1007/BF02685905

- Akkoyun, M.B., Ozdemir, S., Kilinc, E., Birhanli, E., Aygün, A., Sen, F. (2020). Resistance, removal, and bioaccumulation of Ni(II) and Co(II) and their impacts on antioxidant enzymes of Anoxybacillus mongoliensis. Comp Biochem Physiol C. 235:108790. https://doi.org/10.1016/j.cbpc.2020.108790

- Albert, Q., Leleyter, L., Lemoine, M., Heutte, N., Rioult, J.P., Sage, L. (2018). Biosorption of Cd, Cu, Pb by Absidia cylindrospora. Chemosphere. 196:386–392. https://doi.org/10.1016/j.chemosphere.2017.12.158

- Ali J, Ali N, Wang L, Waseem H, Pan G. (2019b). Revisiting the mechanistic pathways for bacterial mediated synthesis of noble metal nanoparticles. J Microbiol Methods. 159:18–25. https://doi.org/10.1016/j.mimet.2019.01.020

- Ali, F, Jilani, G, Fahim, R, Bai, L, Wang, C, Tian, L, Jiang, H. (2019a). Functional roles of rooted macrophytes in nutrient and metal abatement. J Environ Manage. 249:109330. https://doi.org/10.1016/j.jenvman.2019.109330

- Amari, T., Ghnaya, T., Abdelly, C.(2017). Nickel, cadmium and lead phytotoxicity and potential of halophytic plants in heavy metal extraction. S Afr J Bot. 111:99–110. https://doi.org/10.1016/j.sajb.2017.03.011

- Aparicio, J.D., Lacalle, R.G., Artetxe, U., Urionabarrenetxea, E., Becerril, J.M., Polti, M.A., Garbisu, C., Soto, M. (2021). Remediation of soils with chromium and lindane: integration of biological and physico-chemical strategies. Environ Res. 194:110666. https://doi.org/10.1016/j.envres.2020.110666

- Arshadi, M., Yaghmaei, S. (2020). Advances in bioleaching of copper and nickel from electronic waste using Acidithiobacillus ferrooxidans. Chem Pap. 74:2211–2227. https://doi.org/10.1007/s11696-020-01091-4

- Ayogu, C.V., Ifeanyi, V.O., Obasi, N.P. (2020). Monitoring metabolic compounds from degradation of petrochemicals using Pseudomonas consortium. Niger J Microbiol. 34:5221–5238.

- Bae, W., Mehra, R.K., Mulchandani, A., Chen, W. (2001). Genetic engineering of Escherichia coli for enhanced uptake and bioaccumulation of mercury. Appl Environ Microbiol. 67:5335–5338. https://doi.org/10.1128/AEM.67.11.5335-5338.2001

- Bae, W., Wu, C.H., Kostal, J., Mulchandani, A., Chen, W. (2003). Enhanced mercury biosorption by bacterial cells with surface-displayed MerR. Appl Environ Microbiol. 69:3176–3180. https://doi.org/10.1128/AEM.69.6.3176-3180.2003

- Banik, S., Das, K., Islam, M., Salimullah, M. (2014). Microbial bioremediation of heavy metals: recent advancements and challenges. JSM Biotechnol Biomed Eng. 2:1035.

- Bernard, A. (2008). Cadmium and its adverse effects on human health. Indian J Med Res. 128:557–564.

- Brown, B., Immethun, C., Wilkins, M., Saha, R. (2020). Rhodopseudomonas palustris CGA009 polyhydroxybutyrate production from lignin aromatic compounds. Bioresour Technol Rep. 11:100474. https://doi.org/10.1016/j.biteb.2020.100474

- Caracciolo, A.B., Terenzi, V. (2021). Rhizosphere microbial communities and heavy metals. Microorganisms. 9:1462. https://doi.org/10.3390/microorganisms9071462

- Charzyński, P., Ptak, A., Hanaka, A. (2017). Influence of soil sealing on the geoaccumulation index of heavy metals and various pollution factors. Environ Sci Pollut Res. 24:4801–4811. https://doi.org/10.1007/s11356-016-8205-1

- Chaudhary P., Parveen H., Chaudhary A., Ali M., Gautam D. (2022). Scope of antibiotic resistance genes in sewage sludge. In: Sustainable management and utilization of sewage sludge. Springer. p. 227–245. https://doi.org/10.1007/978-3-030-85210-1_10

- Checcucci, A., Bazzicalupo, M., Mengoni, A. (2017). Nitrogen-fixing rhizobial symbionts for phytoremediation improvement. In: Enhancing cleanup of environmental pollutants. Cham: Springer. p. 275–288. https://doi.org/10.1007/978-3-319-55426-6_13

- Dada, O.E. (2019). Cadmium tolerance and phytoremediation strategies of selected tropical plants. West Afr J Appl Ecol. 27:106–125.

- Das N., Vimala R., Karthika P. (2008). Biosorption of heavy metals—an overview. Indian J Biotechnol. 7:159–169.

- Degraeve, N. 1981. Carcinogenic, teratogenic and mutagenic effects of cadmium. Mutat Res. 86:115–135. https://doi.org/10.1016/0165-1110(81)90003-3

- Dixit R., Wasiullah., Malaviya D., Pandiyan K., Singh U.B., Sahu A. (2015). Bioremediation of heavy metals: principles and processes. Sustainability. 7:2189–2212. https://doi.org/10.3390/su7022189

- Edao, H.G. 2017. Heavy metals pollution of soil: toxicity and phytoremediation techniques. Indian J Adv Eng Res Dev. 4:1–10.

- Enebe, M.C., Babalola, O.O. (2018). The influence of plant growth-promoting rhizobacteria in plant tolerance to abiotic stress: a survival strategy. Appl Microbiol Biotechnol. 102:7821–7835. https://doi.org/10.1007/s00253-018-9215-1

- Engwa, G.A., Ferdinand, P.U., Nwalo, F.N., Unachukwu, M.N. (2019). Mechanism and health effects of heavy metal toxicity in humans. In: Poisoning in the modern world: new tricks for an old dog. London: IntechOpen. https://doi.org/10.5772/intechopen.82511

- Etesami, H. (2018). Bacterial mediated alleviation of heavy metal stress and reduced accumulation in plants: mechanisms and prospects. Ecotoxicol Environ Saf. 147:175–191. https://doi.org/10.1016/j.ecoenv.2017.08.037

- Etesami, H., Maheshwari, D.K. (2018). Use of plant growth promoting rhizobacteria in stress agriculture. Ecotoxicol Environ Saf. 156:225–246. https://doi.org/10.1016/j.ecoenv.2018.03.013

- Eyankware, M.O., Obasi, P.N. (2021). A holistic review of heavy metals in water and soil in Ebonyi SE, Nigeria. World News Nat Sci. 38:1–19.

- Fremont, A., Sas, E., Sarrazin, M., Gonzalez, E., Brisson, J., Pitre, F.E., Brereton, N.J.B. (2021). Phytochelatin and coumarin enrichment in root exudates of arsenic-treated white lupin. Plant Cell Environ. 45:936–954. https://doi.org/10.1111/pce.14070

- Gadd, G.M, Bahri-Esfahani, J, Li, Q, Rhee, Y.J, Wei, Z, Fomina, M, Liang, X. (2014). Oxalate production by fungi in geomycology and bioremediation. Fungal Biol Rev. 28:36–55. https://doi.org/10.1016/j.fbr.2014.02.001

- Gamalero, E., Lingua, G., Berta, G., Glick, B.R. (2009). Role of PGPR and AM fungi in plant responses to heavy metal stress. Can J Microbiol. 55:501–514. https://doi.org/10.1139/W09-004

- Gulati, K., Banerjee, B., Lal. S.B, Ray, A. (2010). Effects of diesel exhaust, heavy metals and pesticides on various organ systems: possible mechanisms and strategies for prevention and treatment.

- Gupta S., Singh D. (2017). Role of genetically modified microorganisms in heavy metal bioremediation. In: Kumar R, Sharma AK, Ahluwalia SS, editors. Advances in environmental biotechnology. Singapore: Springer. p. 197–214. https://doi.org/10.1007/978-981-10-1866-4_10

- Hansda, A., Kumar, V. (2017). Cu-resistant Kocuria sp. CRB15: a potential PGPR isolated from the dry tailing of Rakha copper mine. 3 Biotech. 7:132. https://doi.org/10.1007/s13205-017-0747-3

- Hassan, T.U., Bano, A., Naz, I. (2017). Alleviation of heavy metal toxicity by PGPR in wheat grown in saline-sodic soil. Int J Phytoremediation. 19:522–529. https://doi.org/10.1080/15226514.2016.1244163

- Hayat, S., Faraz, A., Faizan, M. (2017). Root exudates: composition and impact on plant–microbe interaction. In: Biofilms in plant and soil health. Wiley. p. 179–193.

- Hemdan, B., Garlapati, V.K., Sharma, S., Bhadra, S., Maddirala, S., Varsha, K.M., Motru, V., Goswami, P., Sevda, S., Aminabhavi, T.M. (2022). Bioelectrochemical systems-based metal recovery: resource conservation and recycling of industrial effluents. Environ Res. 204:112346. https://doi.org/10.1016/j.envres.2021.112346

- Hess, R., Schmid, B. (2002). Zinc supplement overdose can have toxic effects. J Pediatr Hematol Oncol. 24:582–584. https://doi.org/10.1097/00043426-200210000-00018

- Islam, E., Khan, M.T., Irem, S. (2015). Biochemical mechanisms of signaling in plants under arsenic stress. Ecotoxicol Environ Saf. 114:126–133. https://doi.org/10.1016/j.ecoenv.2015.01.023

- Izydorczyk, G., Mikuła, K., Skrzypczak, D., Moustakas, K., Witek-Krowiak, A., Chojnacka, K. (2021). Potential environmental pollution from copper metallurgy and methods of management. Environ Res. 197:111050. https://doi.org/10.1016/j.envres.2021.111050

- Jacobs, I.A., Taddeo, J., Kelly, K., Valenziano, C. (2002). Poisoning as a result of barium styphnate explosion. Am J Ind Med. 41:285–288. https://doi.org/10.1002/ajim.10045

- Jalmi, S.K., Bhagat, P.K., Verma, D., Noryang, S., Tayyeba, S., Singh, K., Sharma, D., Sinha, A.K. (2018). Traversing the links between heavy metal stress and plant signaling. Front Plant Sci. 9:12. https://doi.org/10.3389/fpls.2018.00012

- Jamla M, Khare T, Joshi S, Patil S, Penna S, Kumar V. (2021). Omics approaches for understanding heavy metal responses and tolerance in plants. Curr Plant Biol. 27:100213. https://doi.org/10.1016/j.cpb.2021.100213

- Kaji, M. (2012). Role of experts and public participation in pollution control: the case of Itai-itai disease in Japan. Ethics Sci Environ Polit. 12:99–111. https://doi.org/10.3354/esep00127

- Keiluweit, M., Bougoure, J.J., Nico, P.S., Pett-Ridge, J., Weber, P.K., Kleber, M. (2015). Mineral protection of soil carbon counteracted by root exudates. Nat Clim Chang. 5:588–595. https://doi.org/10.1038/nclimate2580

- Kelly, D.J., Budd, K., Lefebvre, D.D. (2006). Biotransformation of mercury in microfungi cultures. Botany. 84:254–260. https://doi.org/10.1139/B05-158

- Khalid, S., Shahid, M., Niazi, N.K., Murtaza, B., Bibi, I., Dumat, C. (2017). A comparison of technologies for remediation of heavy metal contaminated soils. J Geochem Explor. 182:247–268. https://doi.org/10.1016/j.gexplo.2016.11.021

- Khan, I., Aftab, M., Shakir, S., Ali, M., Qayyum, S., Rehman, M.U. (2019). Mycoremediation of Cd and Cr polluted soil using fungal isolates. Environ Monit Assess. 191:351. https://doi.org/10.1007/s10661-019-7497-x

- Khanna, K., Jamwal, V.L., Sharma, A., Gandhi, S.G., Ohri, P., Bhardwaj, R., Al-Huqail, A.A., Siddiqui, M.H., Ali, H.M., Ahmad, P. (2019). PGPR alleviates cadmium toxicity in Solanum lycopersicum. Chemosphere. 230:628–639. https://doi.org/10.1016/j.chemosphere.2019.05.018

- Kumar, S, Trivedi, PK. (2016). Heavy metal stress signaling in plants. In: Plant metal interaction. Elsevier. p. 585–603. https://doi.org/10.1016/B978-0-12-803158-2.00024-3

- Luo, Q., Wang, S., Sun, L.N., Wang, H. 2017. Metabolic profiling of root exudates from Sedum alfredii. Sci Rep. 7:39878. https://doi.org/10.1038/srep39878

- Luo, S., Yin J., Peng ,Y., Xie J., Wu, H., He D., Li X., Cheng G. (2020). Glutathione in detoxification and symbiosis of Mesorhizobium huakuii. Curr Microbiol. 77:1–10. https://doi.org/10.1007/s00284-019-01784-8

- Ma, Y., Oliveira, R.S., Freitas, H., Zhang, C. (2016). Biochemical and molecular mechanisms of plant–microbe–metal interactions: relevance for phytoremediation. Front Plant Sci. 7:918. https://doi.org/10.3389/fpls.2016.00918

- Miller, R.M. 1995. Biosurfactant-facilitated remediation of metal-contaminated soils. Environ Health Perspect. 103:59–62. https://doi.org/10.1289/ehp.95103s159

- Mishra, J., Singh, R., Arora, N.K. (2017). Alleviation of heavy metal stress in plants and remediation of soil by rhizosphere microorganisms. Front Microbiol. 8:1706. https://doi.org/10.3389/fmicb.2017.01706

- Mishra, P., Qidwai, A., Singh, N., Dikshit, A., Agrawal, S.C. (2021). Heavy metal toxicity in plants: impact, signaling and remediation strategies. In: Heavy metal toxicity in plants. Boca Raton (FL): CRC Press. p. 11–26.

- Oladipo, O.G., Awotoye, O.O., Olayinka, A., Bezuidenhout, C.C., Maboeta, M.S. (2018). Heavy metal tolerance traits of fungi from mining sites. Braz J Microbiol. 49:29–37. https://doi.org/10.1016/j.bjm.2017.06.003

- Orisakwe, O.E., Frazzoli, C., Ilo, C.E., Oritsemuelebi B.(2019). Public health burden of e-waste in Africa. J Health Pollut. 9:22. https://doi.org/10.5696/2156-9614-9.22.190310

- Parihar, M., Rakshit, A., Chitara, M.K., Jatav, H.S., Rajput, V.D., Singh, A.K., Rana, K., Jatav, S.S, Anjum, M., Minkina, T., Kumar, U. (2021). Role of plant-associated microbes in phytoremediation of heavy metal polluted soils. In: Heavy metal toxicity in plants: physiological and molecular adaptations. Cham: Springer. p. 157. https://doi.org/10.1007/978-3-030-79525-5_7

- Parmar, S., Daki, S., Bhattacharya, S., Shrivastav, A. (2022). Microorganisms: an ecofriendly tool for waste management and environmental safety. In: Developments in wastewater treatment research and processes. Elsevier. p. 175–193. https://doi.org/10.1016/B978-0-323-85174-7.00009-3

- Pires, C., Franco, A.R., Pereira, S.I., Henriques, I., Correia, A., Magan, N., Castro, P.M. (2017). Metal (loid)-contaminated soils as a source of culturable heterotrophic aerobic bacteria. Geomicrobiol J. 34:760–768. https://doi.org/10.1080/01490451.2016.1276285

- Prabhakaran, P., Ashraf, M.A., Aqma, W.S. (2016). Microbial stress response to heavy metals in the environment. RSC Adv. 6:109862–109877. https://doi.org/10.1039/C6RA21966G

- Pratush, A., Kumar, A., Hu, Z. (2018). Adverse effect of heavy metals (As, Pb, Hg, and Cr) on health and their bioremediation strategies: a review. Int Microbiol. 21:97–106. https://doi.org/10.1007/s10123-018-0012-3

- Rahman, Z., Singh, V.P. (2019). The relative impact of toxic heavy metals on the total environment: an overview. Environ Monit Assess. 191:419. https://doi.org/10.1007/s10661-019-7528-7

- Rai, K.K, Pandey, N., Meena, R.P., Rai, S.P. (2021). Biotechnological strategies for enhancing heavy metal tolerance in legumes. Ecotoxicol Environ Saf. 208:111750. https://doi.org/10.1016/j.ecoenv.2020.111750

- Raklami, A., Oufdou, K., Tahiri, A.I., Mateos-Naranjo, E., Navarro-Torre, S., Rodríguez-Llorente, I.D., Pajuelo, E. (2019). Safe cultivation of Medicago sativa in metal-polluted soils assisted by PGPR. Microorganisms. 7:212. https://doi.org/10.3390/microorganisms7070212

- Rathoure, A.K., Dhatwalia, V.K. (2016). Toxicity and waste management using bioremediation. Hershey (PA): IGI Global.

- Rehman, A.U., Nazir, S., Irshad, R., Tahir, K., Rehman, K., Islam, R.U., Wahab, Z. (2021). Toxicity of heavy metals in plants and animals and their uptake by magnetic iron oxide nanoparticles. J Mol Liq. 321:114455. https://doi.org/10.1016/j.molliq.2020.114455

- Rezanka, T., Palyzová, A., Faltysová, H., Sigler, K. (2019). Siderophores: metabolites of microorganisms. In: Stud Nat Prod Chem. 60:157–188. Elsevier. https://doi.org/10.1016/B978-0-444-64183-0.00004-7

- Riaz, M., Kamran, M., Fang, Y., Wang, Q., Cao, H., Yang, G., Wang, X. (2021). AM fungi mitigate heavy metal phytotoxicity: a review. J Hazard Mater. 402:123919. https://doi.org/10.1016/j.jhazmat.2020.123919

- Rizvi, A., Zaidi, A., Ameen, F., Ahmed, B., AlKahtani, M.D., Khan, M.S. (2020). Heavy metal induced stress on wheat: phytotoxicity and microbiological management. RSC Adv. 10:38379–38403. https://doi.org/10.1039/D0RA07240C

- Salem, H.M., Eweida, E.A., Farag, A. (2000). Heavy metals in drinking water and their environmental impact on human health. In: Proc Int Conf Environmental Hazards Mitigation. Cairo (Egypt): Cairo University. p. 542–556.

- Saljnikov, E., Lavrishchev, A., Römbke, J., Rinklebe, J., Scherber, C., Wilke, B.M.(2022). Understanding and monitoring chemical and biological soil degradation. In: Advances in understanding soil degradation. Cham: Springer. p. 75–124. https://doi.org/10.1007/978-3-030-70769-2_4

- Saxena, B., Shukla, K., Giri, B. (2017). Arbuscular mycorrhizal fungi and salt stress tolerance. In: Wu QS, editor. Arbuscular mycorrhizas and stress tolerance of plants. Singapore: Springer. p. 67–97. https://doi.org/10.1007/978-981-10-4115-0_4

- Scragg, A.H. (2005). Environmental biotechnology. New York (NY): Oxford University Press.

- Shahid, M., Khalid, S., Abbas, G., Shahid, N., Nadeem, M., Sabir, M. (2015). Heavy metal stress and crop productivity. In: Hakeem KR, editor. Crop production and global environmental issues. Cham: Springer. p. 1–25. https://doi.org/10.1007/978-3-319-23162-4_1

- Sharma, I. (2020). Bioremediation techniques for polluted environment: concepts, advantages, limitations and prospects. In: Trace metals in the environment. London: IntechOpen. https://doi.org/10.5772/intechopen.90453

- Sharma, P., Pandey, A.K., Kim, S.H., Singh, S.P., Chaturvedi, P., Varjani, S. (2021). Microbial community during in situ bioremediation of heavy metals from industrial wastewater: a review. Environ Technol Innov. 24:101826. https://doi.org/10.1016/j.eti.2021.101826

- Shi, S., Li, S., Asim, M., Mao, J., Xu, D., Ullah, Z., Liu, G., Wang, Q., Liu, H. (2018). The Arabidopsis calcium-dependent protein kinases and their roles in plant growth regulation and abiotic stress responses. Int J Mol Sci. 19:1900. https://doi.org/10.3390/ijms19071900

- Singh, J.S., Abhilash, P.C., Singh, H.B., Singh, R.P, Singh, D.P. (2011). Genetically engineered bacteria for environmental remediation: future perspectives. Gene. 480:1–9. https://doi.org/10.1016/j.gene.2011.03.001

- Singh, M., Srivastava, P.K., Verma, P.C., Kharwar, R.N., Singh, N., Tripathi, R.D. (2015). Soil fungi for mycoremediation of arsenic. J Appl Microbiol. 119:1278–1290. https://doi.org/10.1111/jam.12937

- Singh, S., Parihar, P., Singh, R., Singh, V.P., Prasad, S.M. (2016). Heavy metal tolerance in plants: role of transcriptomics, proteomics, metabolomics, and ionomics. Front Plant Sci. 6:1143. https://doi.org/10.3389/fpls.2015.01143

- Sinha, S.N., Paul, D. (2012). Detoxification of heavy metals by biosurfactants. Bull Environ Sci Res. 1:1–3.

- Srivastava, P.K, Vaish, A, Dwivedi, S, Chakrabarty, D, Singh, N, Tripathi, R.D. (2011). Biological removal of arsenic by soil fungi. Sci Total Environ. 409:2430–2442. https://doi.org/10.1016/j.scitotenv.2011.03.002

- Thijs, S., Langill, T., Vangronsveld, J. (2017). The bacterial and fungal microbiota of hyperaccumulator plants. In: Adv Bot Res. 83:43–86. https://doi.org/10.1016/bs.abr.2016.12.003

- Tiwari, S., Lata, C. (2018). Heavy metal stress, signaling, and tolerance due to plant-associated microbes: an overview. Front Plant Sci. 9:452. https://doi.org/10.3389/fpls.2018.00452

- Tripathi R.D, Srivastava S., Mishra S., Singh N., Tuli R., Gupta D.K., Maathuis F.J.M. (2007). Arsenic hazards: strategies for tolerance and remediation by plants. Trends Biotechnol. 25:158–165. https://doi.org/10.1016/j.tibtech.2007.02.003

- Tsyganov, V.E., Tsyganova, A.V., Gorshkov, A.P., Seliverstova, E.V., Kim, V.E., Chizhevskaya, E.P. (2020). Plant–microbe system for cadmium phytoremediation in Pisum sativum. Front Microbiol. 11:15. https://doi.org/10.3389/fmicb.2020.00015

- Tunali, S., Akar, T., Ozcan, A.S., Kiran, I., Ozcan A. (2006). Biosorption of Pb(II) by Cephalosporium aphidicola. Sep Purif Technol. 47:105–112. https://doi.org/10.1016/j.seppur.2005.06.012

- Ullah, A, Heng, S, Munis, MFH, Fahad, S, Yang, X. (2015). Phytoremediation of heavy metals assisted by plant growth promoting bacteria: a review. Environ Exp Bot. 117:28–40. https://doi.org/10.1016/j.envexpbot.2015.05.001

- Vejan, P., Abdullah, R., Khadiran, T., Ismail, S., Nasrulhaq, B. A. (2016). Role of plant growth promoting rhizobacteria in agricultural sustainability. Molecules. 21:573. https://doi.org/10.3390/molecules21050573

- Wu, C., Li, F., Yi, S., Ge, F. (2021). Genetically engineered microbial remediation of co-contaminated soils: advances and ecological risk assessment. J Environ Manage. 296:113185. https://doi.org/10.1016/j.jenvman.2021.113185

- Wu, C.H, Wood, T.K., Mulchandani, A., Chen, W. (2006). Engineering plant–microbe symbiosis for rhizoremediation. Appl Environ Microbiol. 72:1129–1134. https://doi.org/10.1128/AEM.72.2.1129-1134.2006

- Wuana, R.A., Okieimen, F.E. (2011). Heavy metals in contaminated soils: a review of sources, chemistry, risks and best available strategies for remediation. ISRN Ecol. 2011:402647. https://doi.org/10.5402/2011/402647

- Yusuf, M., Fariduddin, Q., Ahmad, A. (2012). 24-Epibrassinolide modulates growth and antioxidant system in Vigna radiata under nickel stress. Plant Physiol Biochem. 57:143–153. https://doi.org/10.1016/j.plaphy.2012.05.011

- Zhang, H., Yuan, X., Xiong, T., Wang, H., Jiang, L. (2020). Bioremediation of co-contaminated soil with heavy metals and pesticides: influence factors, mechanisms and evaluation methods. Chem Eng J. 398:125657. https://doi.org/10.1016/j.cej.2020.125657

- Zoroddu, M.A., Aaseth, J., Crisponi, G., Medici, S., Peana, M., Nurchi, V.M. (2019). The essential metals for humans: a brief overview. J Inorg Biochem. 195:120–129. https://doi.org/10.1016/j.jinorgbio.2019.03.013

- Zubair M., Shakir, M., Ali, Q., Rani, N., Fatima, N., Farooq, S. (2016). Rhizobacteria and phytoremediation of heavy metals. Environ Technol Rev. 5:112–119. https://doi.org/10.1080/21622515.2016.1184173