# Review Article

# Harmful Algal Blooms in Indian Lakes: Causes, Effects, and Community-Level Management

Abstract

Harmful Algal Blooms (HABs) represent a critical environmental, public health, and socio-economic challenge for freshwater ecosystems across India. Driven primarily by anthropogenic nutrient loading, climate change, and altered hydrological regimes, the proliferation of toxic phytoplankton, particularly cyanobacteria, severely degrades lake water quality. This comprehensive analysis explores the multifaceted causes, profound ecological impacts, and localized management strategies for HABs in the context of Indian lakes, such as Dal Lake, Vembanad Lake, Hussain Sagar, and Chilika Lake. Eutrophication, fueled by untreated municipal sewage, industrial effluents, and agricultural runoff laden with nitrogen (NO₃-) and phosphorus (PO₄³-), provides the primary catalyst for explosive algal growth. Consequently, these blooms lead to severe hypoxia, devastating fish kills, and the disruption of complex aquatic food webs. Furthermore, the secretion of potent cyanotoxins, including microcystins and saxitoxins, poses acute health risks to communities relying on these water bodies for drinking, agriculture, and recreation. Given the vast geographical and cultural diversity of India, centralized top-down management approaches often fall short. Therefore, this study emphasizes the necessity of community-level management interventions. By integrating traditional ecological knowledge, low-cost bioremediation techniques like floating wetlands, and active citizen science for early warning monitoring, local populations can effectively mitigate bloom impacts. This paper provides an in-depth synthesis of current research, emphasizing that sustainable lake conservation in India requires a localized, community-driven framework supported by stringent nutrient management policies and continuous scientific monitoring.

Keywords: Harmful Algal Blooms, Eutrophication, Cyanotoxins, Community Management, Indian Lakes.

Introduction

Freshwater lakes in India are invaluable ecological assets, supporting high biodiversity, moderating local microclimates, and serving as the lifeblood for millions of people through agriculture, fisheries, and drinking water supply (Savaliya et al., 2025). Aquaculture has emerged as one of the fastest-growing sectors contributing to aquatic food production and livelihood security, thereby increasing its interaction with natural freshwater ecosystems (Devi et al., 2025). However, rapid urbanization, intensive agricultural practices, and industrial expansion have placed unprecedented stress on these fragile ecosystems. These stressors introduce a wide range of pollutants that can bioaccumulate and exert toxic effects on fish, altering physiological processes and increasing vulnerability to environmental stress (Satkar et al., 2024). In addition, limited adoption of improved aquaculture practices and inadequate management of water quality further exacerbate pressures on aquatic ecosystems (Karale et al., 2025). One of the most visible and destructive consequences of this anthropogenic degradation is the increasing frequency, duration, and intensity of Harmful Algal Blooms (HABs). A harmful algal bloom occurs when colonies of algae or cyanobacteria commonly referred to as blue-green algae grow out of control and produce toxic or harmful effects on people, fish, shellfish, marine mammals, and birds. In the Indian context, major water bodies ranging from the high-altitude Dal Lake in Kashmir to the tropical Vembanad Lake in Kerala, and the heavily urbanized Hussain Sagar in Hyderabad, are continuously battling severe HAB occurrences.

The primary driver of these blooms is eutrophication, a process where water bodies become overly enriched with nutrients, specifically nitrogen and phosphorus compounds. When agricultural runoff carrying synthetic fertilizers, or untreated domestic sewage containing high loads of organic matter, enters a lake, it disrupts the natural nutrient balance. Elevated concentrations of PO₄³⁻ and NO₃⁻ act as fertilizers for microscopic phytoplankton. Under favorable environmental conditions such as the warm temperatures characteristic of the Indian subcontinent and stagnant water columns, these organisms multiply exponentially (Savaliya et al., 2026). Species such as Microcystis aeruginosa, Anabaena flos-aquae, and Cylindrospermopsis raciborskii are frequently identified in Indian lakes, painting the water surface in thick, putrid shades of green, red, or brown.

The implications of HABs extend far beyond mere aesthetic degradation. Deterioration in water quality and environmental stress can also adversely affect fish quality, leading to nutrient loss, textural degradation, and reduced consumer acceptability during storage and processing (Karale et al., 2025). As these massive algal biomasses eventually die and decompose, aerobic bacteria consume vast quantities of dissolved oxygen (O₂), leading to hypoxic or anoxic conditions that trigger catastrophic fish kills and destroy benthic habitats (Savaliya et al., 2026). Furthermore, many bloom-forming cyanobacteria release potent secondary metabolites known as cyanotoxins. These neurotoxins and hepatotoxins endanger human communities that depend on these lakes, causing a spectrum of illnesses from dermal irritation to severe liver damage. Managing this crisis requires moving beyond generic, large-scale infrastructural projects. It demands an urgent shift toward decentralized, community-level management strategies. By empowering local stakeholders with scientific knowledge, citizen science tools, and bioremediation techniques, India can forge a sustainable path to restoring the ecological integrity of its deteriorating lentic ecosystems.

2. The Current Ecological State of India's Lakes and Reservoirs

India's freshwater lakes and reservoirs, numbering over 2,000 significant bodies, are undergoing rapid ecological deterioration, transforming from vital biodiversity hotspots into polluted wastelands dominated by harmful algal blooms. Once teeming with diverse aquatic life and supporting robust fisheries, these lentic ecosystems now exhibit widespread symptoms of eutrophication, characterized by excessive nutrient enrichment, oxygen depletion, and proliferation of toxic phytoplankton. According to recent surveys by the Central Pollution Control Board, approximately 70% of monitored lakes across states like Uttar Pradesh, Maharashtra, and Kerala are classified as eutrophic or hypereutrophic, with total phosphorus levels often exceeding 100 μg/L and total nitrogen surpassing 1 mg/L far above the permissible limits for oligotrophic conditions. This degradation is exacerbated by stagnant water flows, shallow depths in many reservoirs (average 5-10 m), and high ambient temperatures averaging 25-35°C, fostering ideal conditions for cyanobacterial dominance. Visually, lakes present a dystopian spectacle: surfaces cloaked in thick scums of green Microcystis or red Oscillatoria, emitting foul odors from anaerobic decomposition, signaling a profound shift from clear, oxygenated waters to hypoxic "dead zones". The visible manifestations of harmful algal blooms in Indian lakes are illustrated in Figure 1.

Figure 1: The Visual Impact of HABs

Urban lakes bear the brunt of this crisis, serving as sinks for metropolitan waste. The introduction and expansion of aquaculture species, particularly non-native fishes, can further alter aquatic ecosystems by modifying trophic interactions and increasing nutrient loading (Funde et al., 2025). Hussain Sagar in Hyderabad, a historic urban centerpiece, has recurrently turned into a frothy, algal-choked basin since the 2010s, with dissolved oxygen plummeting below 2 mg/L during summer peaks, triggering mass fish mortalities documented in over 50 events between 2015-2023. Similarly, Bengaluru's Bellandur Lake exemplifies industrial-age pollution, where untreated sewage and solid waste ignite spontaneous foaming from surfactants and phosphates, rendering 90% of its 3.5 km² surface unusable for recreation or irrigation. Dal Lake in Srinagar, shrunk from 22 km² in the 1970s to under 10 km² today due to encroachment and siltation, now hosts persistent blooms of Anabaena, reducing water transparency to less than 0.5 m and devastating its shikara-based tourism economy. These cases underscore a national pattern: urban lakes exhibit biochemical oxygen demand levels routinely above 20 mg/L, violating CPCB standards by factors of 10 or more.

In contrast, peri-urban and rural reservoirs reveal subtler yet equally alarming declines linked to agricultural intensification. Chilika Lake, Asia's largest brackish lagoon spanning 1,100 km² in Odisha, has seen HAB incursions intensify post-2000 due to altered hydrology from upstream dams, with Microcystis blooms covering up to 30% of its area during monsoons, correlating with a 40% drop in shrimp and fish landings. Vembanad Lake in Kerala, integral to the backwater fishery yielding 50,000 tons annually, now grapples with year-round cyanobacterial mats, driven by coconut husk retting and fertilizer runoff, resulting in DO minima of 1-3 mg/L and pH swings from algal photosynthesis. Nationwide, reservoir surveys indicate a 25-50% reduction in macrophyte cover natural oxygenators replaced by invasive floating species like Eichhornia crassipes (water hyacinth), which further stifle light penetration and exacerbate anoxia.

The biodiversity toll is staggering: native fish species diversity has declined by 30-60% in affected lakes, with endemic cyprinids vanishing amid hypoxic events that claim millions of fish yearly. These declines are often associated with cumulative physiological stress and tissue-level damage in fish exposed to suboptimal water quality conditions (Raval et al., 2025). Benthic macroinvertebrates, key indicators of ecosystem health, show 80% reductions in polluted sites, as measured by biotic indices like the Shannon-Wiener diversity (H' <1.0). Zooplankton communities shift toward tolerant rotifers, disrupting food webs and cascading to avian populations migratory birds at Chilika have halved since 1990s peaks. Such ecological imbalances are often exacerbated by external nutrient inputs from human activities, including aquaculture practices that alter trophic dynamics (Halpati et al., 2026). These ecological imbalances herald broader consequences, yet they also spotlight intervention windows through localized monitoring.

This dire state demands immediate reckoning: India's lakes are at a tipping point, where unchecked nutrient fluxes and warming trends could render 80% unusable by 2030, per predictive models. Transitioning toward sustainable aquaculture practices, including alternative feed resources and circular bioeconomy approaches, is essential to reduce environmental degradation and ensure long-term ecosystem resilience (Bhusare et al., 2026). Transitioning to resilient management hinges on quantifying these stressors eutrophication foremost before irreversible collapse. This process is fundamentally driven by anthropogenic nutrient inputs, specifically nitrogen and phosphorus from untreated sewage and agricultural runoff, which stimulate excessive phytoplankton biomass and degrade water quality (Kumar & Mahajan, 2020; Nair & Nayak, 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hdnlhIFZpa3JhbWFuIiwiZmFtaWx5IjoiTmFpciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQcmF0ZWVwIEt1bWFyIiwiZmFtaWx5IjoiTmF5YWsifX1dLCJpZHMiOnsiaXNzbiI6WyIyMjk2LTc3NDUiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzODkvZm1hcnMuMjAyMy4xMDg3Mjk2Iiwib3BlbmFsZXgiOiJXNDM4MjcyMzEzMyIsIm9wZW5BY2Nlc3NJZCI6IjEzMTM3MzEwNDMyMTcwOTY3ODE2In0sInRpdGxlIjoiVW5jb3ZlcmluZyB3YXRlciBxdWFsaXR5IGFuZCBldmFsdWF0aW5nIHZ1bG5lcmFiaWxpdGllcyBvZiBzbWFsbC1zY2FsZSBmaXNoZXJpZXMgaW4gQ2hpbGlrYSBMYWdvb24sIEluZGlhIiwiYWJzdHJhY3QiOiJTbWFsbC1zY2FsZSBmaXNoZXJpZXMgKFNTRnMpIHBsYXkgYSBzdHJvbmcgcm9sZSBpbiBzdXN0YWluaW5nIG1pbGxpb25zIG9mIGxpdmVsaWhvb2RzLCBmb29kIHNlY3VyaXR5LCBudXRyaXRpb24sIGFuZCBpbmNvbWUgZ2xvYmFsbHkgYnV0IHRoZSBmaXNoZXJzIGVuZ2FnZWQgaW4gdGhpcyBzZWN0b3Igc2ltdWx0YW5lb3VzbHkgZXhwZXJpZW5jZSBoaWdoIGxldmVscyBvZiB2dWxuZXJhYmlsaXR5IGFuZCBwcm9jZXNzZXMgb2YgbWFyZ2luYWxpc2F0aW9uLiBTZXZlcmFsIGZhY3RvcnMgYXJlIGF0dHJpYnV0ZWQgdG8gdGhlIG11bHRpZGltZW5zaW9uYWwgdnVsbmVyYWJpbGl0aWVzIHRoZSBzbWFsbC1zY2FsZSBmaXNoZXJzIGV4cGVyaWVuY2UsIHNwYW5uaW5nIGJvdGggbmF0dXJhbCAoZS5nLiwgbmF0dXJhbCBkaXNhc3RlcnMsIGVjb3N5c3RlbSBjaGFuZ2UpIGFuZCBhbnRocm9wb2dlbmljIChlLmcuLCBwb2xpY3kgY2hhbmdlLCBoeWRyb2xvZ2ljYWwgaW50ZXJ2ZW50aW9ucywgYXF1YWN1bHR1cmUpIHByZXNzdXJlcy4gV2hpbGUgdGhlcmUgaXMgbXVjaCBsaXRlcmF0dXJlIG9uIHZhcmlvdXMgbmF0dXJhbCBhbmQgaHVtYW4gZHJpdmVycyBvZiB2dWxuZXJhYmlsaXR5IGluIHNtYWxsLXNjYWxlIGZpc2hlcnkgY29tbXVuaXRpZXMsIGFuIGFic2VuY2Ugb2YgcmVzZWFyY2ggY29ubmVjdGluZyB2dWxuZXJhYmlsaXR5IHdpdGggd2F0ZXIgcXVhbGl0eSBpcyBldmlkZW50LiBGaXNoZXIgY29tbXVuaXRpZXMgb2Z0ZW4gdGFsayBhYm91dCBmaXNoIGluIHJlbGF0aW9uIHRvIHRoZSBoZWFsdGggb2YgdGhlaXIgYXF1YXRpYyBoYWJpdGF0cyB3aGVyZWluIHdhdGVyIHF1YWxpdHkgaXMgc2VlbiBhcyBhIGtleSBwYXJhbWV0ZXIuIFRoZSBsaW5rIGJldHdlZW4gaGVhbHRoeSBmaXNoIGFuZCBnb29kIHF1YWxpdHkgd2F0ZXIgaGFzIHNpZ25pZmljYW50IGltcGxpY2F0aW9ucyBmb3Igc3Ryb25nIGFuZCB2aWFibGUgZmlzaGluZyBjb21tdW5pdGllcy4gVGhpcyBwYXBlciBleGFtaW5lcyB0aGVzZSBsaW5rcyBmdXJ0aGVyIGJ5IGZvY3VzaW5nIG9uIHRoZSBuYXR1cmUgb2YgdnVsbmVyYWJpbGl0aWVzIGNhdXNlZCBieSB3YXRlciBxdWFsaXR5IGNoYW5nZXMgaW4gdGhlIHNtYWxsLXNjYWxlIGZpc2hlcnkgc3lzdGVtIG9mIENoaWxpa2EgTGFnb29uIGluIEluZGlhLiBXZSB1bmRlcnRha2UgZGV0YWlsZWQgYW5hbHlzaXMgb2YgdGhlIGludmFzaXZlIHNocmltcCBhcXVhY3VsdHVyZSBhY3Rpdml0aWVzIGFuZCBoeWRyb2xvZ2ljYWwgaW50ZXJ2ZW50aW9ucyBmb3Igb3BlbmluZyBvZiBhIGxhZ29vbiBpbmxldCB3aXRoIHRoZSBCYXkgb2YgQmVuZ2FsIGFzIHR3byBkb21pbmFudCBkcml2ZXJzIGFkdmVyc2VseSBpbXBhY3Rpbmcgd2F0ZXIgcXVhbGl0eSBhbmQgaW5jcmVhc2luZyB2dWxuZXJhYmlsaXRpZXMgb2YgdGhlIGVudGlyZSBzbWFsbC1zY2FsZSBmaXNoZXJpZXMgc29jaWFsLWVjb2xvZ2ljYWwgc3lzdGVtLiBPdXIgYW5hbHlzaXMgc3VnZ2VzdHMgdGhhdCB0aGVyZSBhcmUgc3Ryb25nIGludGVyY29ubmVjdGlvbnMgYmV0d2VlbiBjaGFuZ2VzIGluIHdhdGVyIHF1YWxpdHkgYW5kIHRoZSBsZXZlbHMgb2YgdnVsbmVyYWJpbGl0aWVzIGluIHRoZSBTU0ZzIG9mIENoaWxpa2EgTGFnb29uLiBQb2xsdXRhbnRzIHN1Y2ggYXMgcGVzdGljaWRlcywgYW5kIG9yZ2FuaWMgY29tcG91bmRzIGFjY3VtdWxhdGUgaW4gZmlzaCB0aXNzdWVzIGFuZCBhZmZlY3QgdGhlaXIgZ3Jvd3RoLCByZXByb2R1Y3Rpb24sIGFuZCBvdmVyYWxsIGhlYWx0aC4gVGhpcyBsZWQgdG8gZGVjbGluZXMgaW4gZmlzaCBwb3B1bGF0aW9ucywgbWFraW5nIGl0IG1vcmUgZGlmZmljdWx0IGZvciBmaXNoZXJzIHRvIG1ha2UgYSBsaXZpbmcuIEluIGFkZGl0aW9uIHRvIGRpcmVjdCBpbXBhY3RzIG9uIGZpc2ggcG9wdWxhdGlvbnMsIHBvb3Igd2F0ZXIgcXVhbGl0eSBhbHNvIGhhcyBpbmRpcmVjdCBlZmZlY3RzIG9uIHRoZSBzb2NpYWwgYW5kIGVjb25vbWljIHZ1bG5lcmFiaWxpdHkgb2YgU1NGcy4gRm9yIGV4YW1wbGUsIGNvbnRhbWluYXRpb24gb2Ygd2F0ZXIgc291cmNlcyBsZWQgdG8gdGhlIHJlZHVjZWQgbnVtYmVyIG9mIGZpc2ggc3BlY2llcyByZWR1Y2luZyB0aGUgYW1vdW50IG9mIHRpbWUgZmlzaGVycyBjYW4gc3BlbmQgb24gZmlzaGluZyBhY3Rpdml0aWVzLiBUaGlzIGFsc28gYWZmZWN0ZWQgdGhlIG1hcmtldGFiaWxpdHkgb2YgZmlzaCBwcm9kdWN0cywgcmVkdWNlZCBpbmNvbWUgYW5kIGluY3JlYXNlZCBwb3ZlcnR5LiBUbyBmdWxseSB1bmRlcnN0YW5kIHRoZSBpbnRlcmNvbm5lY3Rpb25zIGJldHdlZW4gd2F0ZXIgcXVhbGl0eSBhbmQgdnVsbmVyYWJpbGl0eSBpbiBTU0ZzIGluIENoaWxpa2EgTGFnb29uLCBpdCBpcyBpbXBvcnRhbnQgdG8gY29uc2lkZXIgYm90aCBlbnZpcm9ubWVudGFsIGFuZCBzb2NpYWwgZmFjdG9ycywgYXMgd2VsbCBhcyB0aGUgY29tcGxleCBmZWVkYmFjayBsb29wcyBiZXR3ZWVuIHRoZXNlIGZhY3RvcnMuIFRoZSBzdHVkeSBoZWxwcyBpbiBicmlkZ2luZyBhIGNydWNpYWwgZ2FwIGluIG91ciB1bmRlcnN0YW5kaW5nIG9mIHRoZSByb2xlIG9mIHdhdGVyIHF1YWxpdHkgaW4gdnVsbmVyYWJpbGl0eSBhbmFseXNpcyB3aXRoaW4gcmVzb3VyY2UgZGVwZW5kZW50IGNvbW11bml0aWVzLiBXZSBjb25jbHVkZSB3aXRoIGtleSBpbnNpZ2h0cyBvbiBwb3NzaWJsZSBjb3BpbmcgcmVzcG9uc2VzIGFuZCBhZGFwdGl2ZSBjYXBhY2l0eSBuZWNlc3NhcnkgZm9yIHRoZSBzbWFsbC1zY2FsZSBmaXNoZXJpZXMgY29tbXVuaXRpZXMgdG8gdHJhbnNpdGlvbiB0b3dhcmQgdmlhYmlsaXR5LiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzg5L2ZtYXJzLjIwMjMuMTA4NzI5NiIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cuZnJvbnRpZXJzaW4ub3JnL2FydGljbGVzLzEwLjMzODkvZm1hcnMuMjAyMy4xMDg3Mjk2L3BkZiIsImlzc3VlZERhdGUiOiIyMDIzLTA2LTI5IiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiRnJvbnRpZXJzIGluIE1hcmluZSBTY2llbmNlIiwidm9sdW1lIjoxMCwiaW1wYWN0RmFjdG9yIjoiNC4yOCJ9LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJGcm9udGllcnMgTWVkaWEifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjUxLCJjaXRlZEJ5Q291bnQiOjV9fSwicXVvdGUiOiJXYXRlciBxdWFsaXR5IG9mIENoaWxpa2EgTGFnb29uIGhhcyBkZWdyYWRlZCBkdWUgdG8gYW50aHJvcG9nZW5pYyBpbnRlcnZlbnRpb25zIGFuZCBuYXR1cmFsIGZvcmNlcyBvZiBjaGFuZ2UuIFZhcmlvdXMgYWN0aXZpdGllcyBpbiB0aGUgQ2hpbGlrYSBMYWdvb24sIHN1Y2ggYXMgaW5kdXN0cmlhbCB3YXN0ZXdhdGVyIGRpc3Bvc2FsLCBzZXdhZ2UgZHVtcGluZywgYXF1YWN1bHR1cmUsIGh5ZHJvbG9naWNhbCBpbnRlcnZlbnRpb25zLCBhbmQgY3ljbG9uZXMsIGFyZSBwcm9kdWNpbmcgc2FsaW5pdHkgY2hhbmdlcywgc2VkaW1lbnQgZGVwb3NpdGlvbiwgbnV0cmllbnQgZW5yaWNobWVudCwgZXV0cm9waGljYXRpb24sIGFuZCBkZWFkIHpvbmVzLCBhcyBzdGF0ZWQuIEZpc2ggc3Vydml2YWwgaXMgYXQgcmlzayBhcyBhIHJlc3VsdCBvZiB0aGUgc2lnbmlmaWNhbnQgcmVkdWN0aW9uIGluIGVudmlyb25tZW50YWwgY2lyY3Vtc3RhbmNlcywgd2hpY2ggZGltaW5pc2hlcyBmaXNoZXJtZW7igJlzIGluY29tZSBhbmQgcHVzaGVzIHRoZW0gb3V0IG9mIHNvY2lldHkuIFRoZXNlIGNhdXNlcyBhZmZlY3RpbmcgdGhlIHdhdGVyIHF1YWxpdHkgb2YgdGhlIExhZ29vbiBkZXBpY3QgbWFueSBmYWNlcyBvZiB2dWxuZXJhYmlsaXR5IGluIFNTRiBjb21tdW5pdGllcy4gQXMgdGhlIENoaWxpa2EgTGFnb29uIGVudmlyb25tZW50IGlzIGV4cGVjdGVkIHRvIHVuZGVyZ28gaW5jcmVhc2luZ2x5IGRyYXN0aWMgY2hhbmdlcyAoc3VjaCBhcyBjYXRjaCBjb21wb3NpdGlvbiwgY2F0Y2ggY2FwYWNpdHksIGJpb2RpdmVyc2l0eSB2YXJpYXRpb25zLCB3YXRlciBxdWFsaXR5LCBhbmQgZmlzaGVyeSByZXZlbnVlcyksIGludGVybmF0aW9uYWwgcG9saWN5IGhhcyBiZWNvbWUgbW9yZSBpbnRlcmVzdGVkIGluIGFkYXB0YXRpb24gbWVhc3VyZXMuIEdpdmVuIGhvdyBkaXZlcnNlIFNTRiBlY29zeXN0ZW1zIGFyZSBleHBlY3RlZCB0byBiZSBiZXR3ZWVuIGNvbnRpbmVudHMsIHRoZXJlIGlzIG5vIOKAnG9uZSBzaXplIGZpdHMgYWxs4oCdIGFwcHJvYWNoLiBUaGUgcGFwZXIgb3V0bGluZWQgc2V2ZXJhbCBjb3BpbmcgYW5kIGFkYXB0YXRpb24gbWVjaGFuaXNtcyB0byBpbXByb3ZlIHdhdGVyIHF1YWxpdHkgYW5kIGNvbnRpbnVlIGZpc2hpbmcgaW4gYSBzdXN0YWluYWJsZSB3YXkuIEFkYXB0YXRpb24gc3RyYXRlZ2llcyBjYW4gb2NjYXNpb25hbGx5IGhhdmUgdW5mb3Jlc2VlbiBjb25zZXF1ZW5jZXMgZm9yIGZpc2hlcmllcyBhbmQgZmlzaGluZyBjb21tdW5pdGllcy4gQW55IHVuZXhwZWN0ZWQgY29uc2VxdWVuY2VzIG9mIGFkYXB0YXRpb24gY2FuIGJlIG1pdGlnYXRlZCB0aHJvdWdoIGxvbmctdGVybSBwbGFubmluZyBhbmQgaWRlbnRpZmljYXRpb24gb2YgZnV0dXJlIG9wdGlvbnMgdGhyb3VnaCBzY2VuYXJpbyBhbmFseXNpcy4gVGhlIGZpbmRpbmdzIG9mIHRoaXMgcGFwZXIgY2FuIGJlIHV0aWxpemVkIHRvIGRldmVsb3AgcG9saWNpZXMgdG8gcHJvdGVjdCB3YXRlciBxdWFsaXR5IGFuZCBTU0ZzIGluIGEgdmFyaWV0eSBvZiBjb2FzdGFsIGxhZ29vbiBzZXR0aW5ncy4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjIuMjg1MDM2MDg3MDM2MTMzLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDkzMDUzNDM2Mjc5MywiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjMuNzczNTE5NTE1OTkxMjEsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40ODMzNjQxMDUyMjQ2MSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjUuMjU1NTg0NzE2Nzk2ODc1LCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDg5NTU5MTczNTgzOTg0LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyNi43NDQwNjgxNDU3NTE5NTMsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC41MDY4MDU0MTk5MjE4NzUsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjI4LjIyNjEzMzM0NjU1NzYxNywibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ5MDQ2MzI1NjgzNTk0LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyOS43MTQ2MTY3NzU1MTI2OTUsImxlZnQiOjguMzMxNTkyNTU5ODE0NDUzLCJ3aWR0aCI6NDAuNDk0MDk4NjYzMzMwMDgsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjMxLjE5NjY4Mzg4MzY2Njk5MiwibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ5NDM5MjM5NTAxOTUzLCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozMi42ODUxNjU0MDUyNzM0NCwibGVmdCI6OC4zMzE1OTYzNzQ1MTE3MTksIndpZHRoIjo0MC40ODc0NDU4MzEyOTg4MywiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MzQuMTY3MjMyNTEzNDI3NzM0LCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ5Nzc0NTUxMzkxNjAxNiwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MzUuNjU1NzE1OTQyMzgyODEsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDk4NjM4MTUzMDc2MTcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM3LjEzNzc4MzA1MDUzNzExLCJsZWZ0IjoxMC42ODUxODYzODYxMDgzOTgsIndpZHRoIjozOC4xNDExMDU2NTE4NTU0NywiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MzguNjI2MjY2NDc5NDkyMTksImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDg3MTUyMDk5NjA5Mzc1LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo0MC4xMDgzMzM1ODc2NDY0ODQsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDg5ODU2NzE5OTcwNywiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDEuNTk2ODEzMjAxOTA0MywibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40ODg2NjY1MzQ0MjM4MywiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDMuMDc4ODgwMzEwMDU4NTk0LCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4MTg2MTExNDUwMTk1LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo0NC41NjczNjM3MzkwMTM2NywibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40ODMwNTg5Mjk0NDMzNiwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDYuMDQ5NDMwODQ3MTY3OTcsImxlZnQiOjguMzMzMTA4OTAxOTc3NTM5LCJ3aWR0aCI6NDAuNTAwNzQ3NjgwNjY0MDYsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQ3LjUzNzkxNDI3NjEyMzA1LCJsZWZ0Ijo4LjMzMzEwODkwMTk3NzUzOSwid2lkdGgiOjQwLjQ5NTYwNTQ2ODc1LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo0OS4wMTk5ODEzODQyNzczNDQsImxlZnQiOjguMzMzMTA2MDQwOTU0NTksIndpZHRoIjo0MC41MDY4MTY4NjQwMTM2NywiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NTAuNTA4NDY0ODEzMjMyNDIsImxlZnQiOjguMzMzMTA2MDQwOTU0NTksIndpZHRoIjo0MC40ODg1MDYzMTcxMzg2NywiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NTEuOTkwNTI4MTA2Njg5NDUsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40OTc3MjY0NDA0Mjk2OSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NTMuNDc5MDExNTM1NjQ0NTMsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjozMy4xNTk0NDY3MTYzMDg1OTQsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjU0Ljk2MTA3ODY0Mzc5ODgzLCJsZWZ0IjoxMC42ODM2NzAwNDM5NDUzMTIsIndpZHRoIjozOC4xMzY1OTY2Nzk2ODc1LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo1Ni40NDk1NjIwNzI3NTM5MDYsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40ODcxNDQ0NzAyMTQ4NDQsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX1dfSwiY2h1bmtJZCI6IjRjZjk1OWQ2LTRiYTQtNDM4Ny1iZjg4LTc4YzMxMGNiZWRmYyIsImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEzIn19LHsib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlBhd2FuIiwiZmFtaWx5IjoiS3VtYXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW1icmlzaCBLdW1hciIsImZhbWlseSI6Ik1haGFqYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyNTIzLTM5NjMiLCIyNTIzLTM5NzEiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMDcvczQyNDUyLTAyMC0zMDgyLTgiLCJtYWciOiIzMDM1OTYwMjI4Iiwib3BlbmFsZXgiOiJXMzAzNTk2MDIyOCIsIm9wZW5BY2Nlc3NJZCI6IjYyMDM0MDYwMTcyODYyMzcwMDIifSwidGl0bGUiOiJUcm9waGljIHN0YXR1cyBhbmQgaXRzIHJlZ3VsYXRpbmcgZmFjdG9yIGRldGVybWluYXRpb24gYXQgdGhlIFJld2Fsc2FyIExha2UsIG5vcnRod2VzdCBIaW1hbGF5YSAoSFApLCBJbmRpYSwgYmFzZWQgb24gc2VsZWN0ZWQgcGFyYW1ldGVycyBhbmQgbXVsdGl2YXJpYXRlIHN0YXRpc3RpY2FsIGFuYWx5c2lzIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjEwMDcvczQyNDUyLTAyMC0zMDgyLTgiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vbGluay5zcHJpbmdlci5jb20vY29udGVudC9wZGYvMTAuMTAwNy9zNDI0NTItMDIwLTMwODItOC5wZGYiLCJpc3N1ZWREYXRlIjoiMjAyMC0wNi0yMiIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IlNOIEFwcGxpZWQgU2NpZW5jZXMiLCJpc3N1ZSI6Nywidm9sdW1lIjoyLCJpbXBhY3RGYWN0b3IiOiI3LjM1In0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlNwcmluZ2VyIE5hdHVyZSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NDAsImNpdGVkQnlDb3VudCI6MTl9fSwicXVvdGUiOiJDaGFuZ2luZyBxdWFsaXR5IGhhcyBjcmVhdGVkIHRoZSB3YXRlciBzY2FyY2l0eSBhbmQgIGEgaHVnZSBtYXNzIG9mIHBvcHVsYXRpb24gdGh1cyBiZWVuIGRlcHJpdmVkIGZyb20gdXNpbmcgIHRoZSBmcmVzaHdhdGVyIGZvciB2YXJpb3VzIHB1cnBvc2VzIFszLCA0XS4gSG93ZXZlciwgdGhlICBvYm5veGlvdXMgbWF0ZXJpYWxzIHRoYXQgc3RheSBpbiB3YXRlciBib2RpZXMgZm9yIGEgbG9uZy10aW1lIHBlcmlvZCBicmluZyBjaGFuZ2VzIGluIHRoZSBwaHlzaW9jaGVtaWNhbCBjaGFyLWFjdGVyaXN0aWNzIGFzIHdlbGwgYXMgdGhlIG5hdHVyYWwgc2NlbmFyaW8gb2YgdGhlIGxha2VzIFs1XS4gIFRoZSBIaW1hbGF5YW4gbGFrZXMgYXJlIHVuZGVyIHN0cmVzcyBkdWUgdG8gbWlzbWFuYWdlZCBodW1hbiBhY3Rpdml0aWVzLCB3aGljaCBpbmNyZWFzZSBudXRyaWVudHMgYW5kIHBvbGx1dGlvbiBpbiB0ZXJtcyBvZiBoZWF2eSBtZXRhbHMgdG94aWNpdHkgYW5kIGJhY3RlcmlhbCBjb250YW1pbmF0aW9uICBbNuKAkzhdLiBUaGUgbWFuYWdlbWVudCBhbmQgcHJlc2VydmF0aW9uIG9mIHRoZSBsYWtlcyBhdCAgcHJlc2VudCBoYXZlIGJlY29tZSBhIGdyZWF0IGNoYWxsZW5nZSBmb3IgZW52aXJvbm1lbnRhbC1pc3RzIGFuZCBtYW5hZ2VycyBbNSwgOV0uIFRoZSBldXRyb3BoaWNhdGlvbiBpcyBhbHNvIGEgbWFpbiBwcm9ibGVtIG9mIHRoZSAgZnJlc2h3YXRlciBsYWtlcywgYW5kIGl0IGlzIHNwZWNpZmllZCBmb3IgdGhlIGxha2VzIGhhdmluZyAgbnV0cmllbnQgZW5yaWNobWVudC4gSXQgY2F1c2VzIGV4Y2Vzc2l2ZSBncm93dGggb2YgYWxnYWUgIGFuZCBpcyBjb25zaWRlcmVkIHRvIGJlIHJlc3BvbnNpYmxlIGZvciBkZWdyYWRpbmcgdGhlICBxdWFsaXRhdGl2ZSBuYXR1cmUgb2YgdGhlIGxha2VzIFs1LCAxMF0uIFRoZSBldXRyb3BoaWNhdGlvbiAgaXMgZm9sbG93ZWQgYnkgdGhlIGFsZ2FsIGJsb29tLCB3aGljaCBnZW5lcmF0ZXMgdG94aW4sICB3b3JzZW5zIHdhdGVyIHF1YWxpdHksIGRlcGxldGVzIGRpc3NvbHZlZCBveHlnZW4gKERPKSAqIFBhd2FuIEt1bWFyLCBwYXdhbi5ldnNAZ21haWwuY29tIHwgMURlcGFydG1lbnQgb2YgRW52aXJvbm1lbnRhbCBTY2llbmNlcywgQ2VudHJhbCBVbml2ZXJzaXR5IG9mIEhpbWFjaGFsIFByYWRlc2gsIERoYXJhbXNoYWxhIDE3NjIxNSwgSW5kaWEuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NjguNTc4NDYwNjkzMzU5MzgsImxlZnQiOjguNTcxNDE4NzYyMjA3MDMxLCJ3aWR0aCI6NDAuMzI3NTE0NjQ4NDM3NSwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjcwLjE1ODk5NjU4MjAzMTI1LCJsZWZ0Ijo4LjU2OTczODM4ODA2MTUyMywid2lkdGgiOjQwLjM1NDcyNDg4NDAzMzIsImhlaWdodCI6MS41MTk4NTUyNjA4NDg5OTl9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo3MS43Mzk1NDc3Mjk0OTIxOSwibGVmdCI6OC41Njk3MzgzODgwNjE1MjMsIndpZHRoIjo0MC4zNDk2OTMyOTgzMzk4NDQsImhlaWdodCI6MS41MTk4NTUyNjA4NDg5OTl9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo3My4zMjAwOTEyNDc1NTg2LCJsZWZ0Ijo4LjU2ODA1OTkyMTI2NDY0OCwid2lkdGgiOjQwLjAyMDA4ODE5NTgwMDc4LCJoZWlnaHQiOjEuNTE5ODU1MjYwODQ4OTk5fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NzQuOTAwNjQyMzk1MDE5NTMsImxlZnQiOjguNTY4MDU5OTIxMjY0NjQ4LCJ3aWR0aCI6MzkuOTk5OTI3NTIwNzUxOTUsImhlaWdodCI6MS41MTk4NTUyNjA4NDg5OTl9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo3Ni40ODExODU5MTMwODU5NCwibGVmdCI6OC41NjgwNTcwNjAyNDE3LCJ3aWR0aCI6NDAuMzU3MDc0NzM3NTQ4ODMsImhlaWdodCI6MS41MTk4NTUyNjA4NDg5OTl9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo3OC4wNjE3Mjk0MzExNTIzNCwibGVmdCI6OC41Njk3MzY0ODA3MTI4OSwid2lkdGgiOjQwLjM1MzcxMzk4OTI1NzgxLCJoZWlnaHQiOjEuNTE5ODU1MjYwODQ4OTk5fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NjAuNjc1NzI3ODQ0MjM4MjgsImxlZnQiOjUxLjQyNzE2NTk4NTEwNzQyLCJ3aWR0aCI6NDAuMzc4NTY2NzQxOTQzMzYsImhlaWdodCI6MTguOTA1ODU1MTc4ODMzMDA4fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6ODEuOTI3MzM3NjQ2NDg0MzgsImxlZnQiOjguNDAxNDk3ODQwODgxMzQ4LCJ3aWR0aCI6NzMuOTU0Njk2NjU1MjczNDQsImhlaWdodCI6Mi42Nzg3NjI2NzQzMzE2NjV9fV19LCJjaHVua0lkIjoiMTcxNzE0MTktY2NmYy00NmJkLWI2ZWYtZjkxNjNkYzRjMTY1IiwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiMSJ9fV0%3D. This nutrient saturation initiates a cascade of ecological disturbances, including severe oxygen depletion, loss of biodiversity, and the proliferation of toxic algal species (Sitote & Gebremedhine, 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllhYnNyYSBNZWxhayIsImZhbWlseSI6IlNpdG90ZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNaWNoYWVsIEdpcmltYXkiLCJmYW1pbHkiOiJHZWJyZW1lZGhpbmUifX1dLCJpZHMiOnsiaXNzbiI6W10sImlzYm4iOltdLCJkb2kiOiIxMC4yMDk0NC9wcmVwcmludHMyMDI0MDkuMDcwNC52MSIsIm9wZW5hbGV4IjoiVzQ0MDI0NjU0MTQiLCJleGEiOiJodHRwczovL3d3dy5wcmVwcmludHMub3JnL21hbnVzY3JpcHQvMjAyNDA5LjA3MDQifSwidGl0bGUiOiJDb21wcmVoZW5zaXZlIFJldmlldyBvZiBFdXRyb3BoaWNhdGlvbiBpbiBGcmVzaHdhdGVyIEVjb3N5c3RlbXM6IENhdXNlcywgRWZmZWN0cywgQXNzZXNzbWVudCwgYW5kIE1hbmFnZW1lbnQgU3RyYXRlZ2llcyIsImFic3RyYWN0IjoiVGhpcyByZXZpZXcgZXhhbWluZXMgZXV0cm9waGljYXRpb24gaW4gZnJlc2h3YXRlciBlY29zeXN0ZW1zLCBhIHdpZGVzcHJlYWQgZW52aXJvbm1lbnRhbCBpc3N1ZSBzdGVtbWluZyBmcm9tIGV4Y2Vzc2l2ZSBudXRyaWVudCBpbnB1dCwgbGFyZ2VseSBkdWUgdG8gaHVtYW4gYWN0aXZpdGllcy4gVGhlIHN0dWR5IGV4cGxvcmVzIHRoZSBvcmlnaW5zLCBjb25zZXF1ZW5jZXMsIGV2YWx1YXRpb24gdGVjaG5pcXVlcywgYW5kIGNvbnRyb2wgbWV0aG9kcyBmb3IgZXV0cm9waGljYXRpb24uIFByaW1hcnkgY2F1c2VzIGluY2x1ZGUgYWdyaWN1bHR1cmFsIGRpc2NoYXJnZSwgdXJiYW4gZGV2ZWxvcG1lbnQsIGFuZCBjbGltYXRlIHNoaWZ0cy4gVGhlIGVmZmVjdHMgYXJlIGZhci1yZWFjaGluZywgaW1wYWN0aW5nIGJpb2RpdmVyc2l0eSwgd2F0ZXIgcXVhbGl0eSwgYW5kIHNvY2lvLWVjb25vbWljIGZhY3RvcnMuIFRoZSBhc3Nlc3NtZW50IHV0aWxpemVzIGEgY29tYmluYXRpb24gb2YgY2hlbWljYWwsIHBoeXNpY2FsLCBhbmQgYmlvbG9naWNhbCBtYXJrZXJzLCB3aXRoIGN1dHRpbmctZWRnZSB0ZWNobm9sb2dpZXMgbGlrZSByZW1vdGUgc2Vuc2luZyBpbXByb3ZpbmcgbW9uaXRvcmluZyBlZmZvcnRzLiBNaXRpZ2F0aW9uIHN0cmF0ZWdpZXMgYXJlIGRpdmVyc2UsIGluY29ycG9yYXRpbmcgcHJldmVudGF0aXZlIGFjdGlvbnMgKHN1Y2ggYXMgbnV0cmllbnQgbGltaXRhdGlvbiBhbmQgbGFuZCBtYW5hZ2VtZW50KSwgY29ycmVjdGl2ZSBtZWFzdXJlcyAoaW5jbHVkaW5nIGJpby1tYW5pcHVsYXRpb24gYW5kIGNoZW1pY2FsIGFwcGxpY2F0aW9ucyksIGFuZCByZWd1bGF0b3J5IGluaXRpYXRpdmVzIChsaWtlIHdhdGVyIHF1YWxpdHkgZ3VpZGVsaW5lcyBhbmQgbnV0cmllbnQgZXhjaGFuZ2Ugc2NoZW1lcykuIFdoaWxlIGNhc2Ugc3R1ZGllcyBkZW1vbnN0cmF0ZSBzdWNjZXNzZnVsIGludGVydmVudGlvbnMsIHRoZXkgYWxzbyByZXZlYWwgb25nb2luZyBjaGFsbGVuZ2VzLCBwYXJ0aWN1bGFybHkgZWNvc3lzdGVtIHVucHJlZGljdGFiaWxpdHkgYW5kIGNsaW1hdGUgY2hhbmdlIGluZmx1ZW5jZXMuIFRoZSByZXNlYXJjaCBoaWdobGlnaHRzIHRoZSBwcm9taXNlIG9mIGVtZXJnaW5nIHRlY2hub2xvZ2llcywgbm90YWJseSBtYWNoaW5lIGxlYXJuaW5nLCBpbiBlbmhhbmNpbmcgbWFuYWdlbWVudCBwcmFjdGljZXMuIEtub3dsZWRnZSBnYXBzIHBlcnNpc3QsIGVzcGVjaWFsbHkgcmVnYXJkaW5nIGV1dHJvcGhpY2F0aW9uLWJpb2RpdmVyc2l0eSBkeW5hbWljcyBpbiB0cm9waWNhbCBlbnZpcm9ubWVudHMgYW5kIHRoZSBzb2Npby1lY29ub21pYyByYW1pZmljYXRpb25zIG9mIGludGVydmVudGlvbnMuIFRoZSBzdHVkeSBlbXBoYXNpemVzIHRoZSBpbXBvcnRhbmNlIG9mIGZsZXhpYmxlLCBjb21wcmVoZW5zaXZlIGFwcHJvYWNoZXMgdGhhdCBibGVuZCBzY2llbnRpZmljIGluc2lnaHRzIHdpdGggcHJhY3RpY2FsIHNvbHV0aW9ucyBhbmQgc3Ryb25nIHBvbGljeSBzdHJ1Y3R1cmVzIHRvIHRhY2tsZSB0aGlzIGludHJpY2F0ZSBlbnZpcm9ubWVudGFsIGNoYWxsZW5nZSBpbiBhbiBlcmEgb2YgY2xpbWF0ZSBjaGFuZ2UuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjIwOTQ0L3ByZXByaW50czIwMjQwOS4wNzA0LnYxIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjA5NDQvcHJlcHJpbnRzMjAyNDA5LjA3MDQudjEiLCJpc3N1ZWREYXRlIjoiMjAyNC0wOS0xMCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTEiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IlByZXByaW50cy5vcmciLCJpbXBhY3RGYWN0b3IiOiIxLjE1In0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjowLCJjaXRlZEJ5Q291bnQiOjR9fSwicXVvdGUiOiJUaGUgc3R1ZHkgZW1waGFzaXplcyB0aGUgaW1wb3J0YW5jZSBvZiBmbGV4aWJsZSwgY29tcHJlaGVuc2l2ZSBhcHByb2FjaGVzIHRoYXQgYmxlbmQgc2NpZW50aWZpYyBpbnNpZ2h0cyB3aXRoIHByYWN0aWNhbCBzb2x1dGlvbnMgYW5kIHN0cm9uZyBwb2xpY3kgc3RydWN0dXJlcyB0byB0YWNrbGUgdGhpcyBpbnRyaWNhdGUgZW52aXJvbm1lbnRhbCBjaGFsbGVuZ2UgaW4gYW4gZXJhIG9mIGNsaW1hdGUgY2hhbmdlLiBFdXRyb3BoaWNhdGlvbiwgY2F1c2VkIGJ5IGV4Y2Vzc2l2ZSBudXRyaWVudCBlbnJpY2htZW50IGluIHdhdGVyIGJvZGllcywgaXMgYSBzaWduaWZpY2FudCBlbnZpcm9ubWVudGFsIGNoYWxsZW5nZSBmb3IgZnJlc2h3YXRlciBlY29zeXN0ZW1zIG9uIGEgZ2xvYmFsIHNjYWxlLiBQcmltYXJpbHkgZHJpdmVuIGJ5IG5pdHJvZ2VuIGFuZCBwaG9zcGhvcnVzIGlucHV0cywgdGhpcyBwcm9jZXNzIHJlc3VsdHMgaW4gdGhlIG92ZXJwcm9kdWN0aW9uIG9mIGFsZ2FlIGFuZCBhcXVhdGljIHBsYW50cywgbGVhZGluZyB0byBhIGNhc2NhZGUgb2YgZWNvbG9naWNhbCBkaXN0dXJiYW5jZXMsIGluY2x1ZGluZyBveHlnZW4gZGVwbGV0aW9uIChoeXBveGlhKSwgbG9zcyBvZiBiaW9kaXZlcnNpdHksIGFuZCBzZXZlcmUgd2F0ZXIgcXVhbGl0eSBkZWdyYWRhdGlvbiAoS2FrYWRlIGV0IGFsLiwgMjAyMTsgTWVlcmhvZmYgZXQgYWwuLCAyMDIyOyBXdXJ0c2JhdWdoIGV0IGFsLiwgMjAxOSkuIiwiZXh0ZXJuYWxDYWNoZUlkIjoiODcwMTIzNzktYzJkYi0zODAwLTkwYzktZWI4NzZjNTEzNjFlIn1d. These blooms are frequently dominated by toxin-producing cyanobacteria such as Microcystis aeruginosa, which release microcystins that compromise potable water safety and necessitate expensive treatment protocols (Sitote & Gebremedhine, 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllhYnNyYSBNZWxhayIsImZhbWlseSI6IlNpdG90ZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNaWNoYWVsIEdpcmltYXkiLCJmYW1pbHkiOiJHZWJyZW1lZGhpbmUifX1dLCJpZHMiOnsiaXNzbiI6W10sImlzYm4iOltdLCJkb2kiOiIxMC4yMDk0NC9wcmVwcmludHMyMDI0MDkuMDcwNC52MSIsIm9wZW5hbGV4IjoiVzQ0MDI0NjU0MTQiLCJleGEiOiJodHRwczovL3d3dy5wcmVwcmludHMub3JnL21hbnVzY3JpcHQvMjAyNDA5LjA3MDQifSwidGl0bGUiOiJDb21wcmVoZW5zaXZlIFJldmlldyBvZiBFdXRyb3BoaWNhdGlvbiBpbiBGcmVzaHdhdGVyIEVjb3N5c3RlbXM6IENhdXNlcywgRWZmZWN0cywgQXNzZXNzbWVudCwgYW5kIE1hbmFnZW1lbnQgU3RyYXRlZ2llcyIsImFic3RyYWN0IjoiVGhpcyByZXZpZXcgZXhhbWluZXMgZXV0cm9waGljYXRpb24gaW4gZnJlc2h3YXRlciBlY29zeXN0ZW1zLCBhIHdpZGVzcHJlYWQgZW52aXJvbm1lbnRhbCBpc3N1ZSBzdGVtbWluZyBmcm9tIGV4Y2Vzc2l2ZSBudXRyaWVudCBpbnB1dCwgbGFyZ2VseSBkdWUgdG8gaHVtYW4gYWN0aXZpdGllcy4gVGhlIHN0dWR5IGV4cGxvcmVzIHRoZSBvcmlnaW5zLCBjb25zZXF1ZW5jZXMsIGV2YWx1YXRpb24gdGVjaG5pcXVlcywgYW5kIGNvbnRyb2wgbWV0aG9kcyBmb3IgZXV0cm9waGljYXRpb24uIFByaW1hcnkgY2F1c2VzIGluY2x1ZGUgYWdyaWN1bHR1cmFsIGRpc2NoYXJnZSwgdXJiYW4gZGV2ZWxvcG1lbnQsIGFuZCBjbGltYXRlIHNoaWZ0cy4gVGhlIGVmZmVjdHMgYXJlIGZhci1yZWFjaGluZywgaW1wYWN0aW5nIGJpb2RpdmVyc2l0eSwgd2F0ZXIgcXVhbGl0eSwgYW5kIHNvY2lvLWVjb25vbWljIGZhY3RvcnMuIFRoZSBhc3Nlc3NtZW50IHV0aWxpemVzIGEgY29tYmluYXRpb24gb2YgY2hlbWljYWwsIHBoeXNpY2FsLCBhbmQgYmlvbG9naWNhbCBtYXJrZXJzLCB3aXRoIGN1dHRpbmctZWRnZSB0ZWNobm9sb2dpZXMgbGlrZSByZW1vdGUgc2Vuc2luZyBpbXByb3ZpbmcgbW9uaXRvcmluZyBlZmZvcnRzLiBNaXRpZ2F0aW9uIHN0cmF0ZWdpZXMgYXJlIGRpdmVyc2UsIGluY29ycG9yYXRpbmcgcHJldmVudGF0aXZlIGFjdGlvbnMgKHN1Y2ggYXMgbnV0cmllbnQgbGltaXRhdGlvbiBhbmQgbGFuZCBtYW5hZ2VtZW50KSwgY29ycmVjdGl2ZSBtZWFzdXJlcyAoaW5jbHVkaW5nIGJpby1tYW5pcHVsYXRpb24gYW5kIGNoZW1pY2FsIGFwcGxpY2F0aW9ucyksIGFuZCByZWd1bGF0b3J5IGluaXRpYXRpdmVzIChsaWtlIHdhdGVyIHF1YWxpdHkgZ3VpZGVsaW5lcyBhbmQgbnV0cmllbnQgZXhjaGFuZ2Ugc2NoZW1lcykuIFdoaWxlIGNhc2Ugc3R1ZGllcyBkZW1vbnN0cmF0ZSBzdWNjZXNzZnVsIGludGVydmVudGlvbnMsIHRoZXkgYWxzbyByZXZlYWwgb25nb2luZyBjaGFsbGVuZ2VzLCBwYXJ0aWN1bGFybHkgZWNvc3lzdGVtIHVucHJlZGljdGFiaWxpdHkgYW5kIGNsaW1hdGUgY2hhbmdlIGluZmx1ZW5jZXMuIFRoZSByZXNlYXJjaCBoaWdobGlnaHRzIHRoZSBwcm9taXNlIG9mIGVtZXJnaW5nIHRlY2hub2xvZ2llcywgbm90YWJseSBtYWNoaW5lIGxlYXJuaW5nLCBpbiBlbmhhbmNpbmcgbWFuYWdlbWVudCBwcmFjdGljZXMuIEtub3dsZWRnZSBnYXBzIHBlcnNpc3QsIGVzcGVjaWFsbHkgcmVnYXJkaW5nIGV1dHJvcGhpY2F0aW9uLWJpb2RpdmVyc2l0eSBkeW5hbWljcyBpbiB0cm9waWNhbCBlbnZpcm9ubWVudHMgYW5kIHRoZSBzb2Npby1lY29ub21pYyByYW1pZmljYXRpb25zIG9mIGludGVydmVudGlvbnMuIFRoZSBzdHVkeSBlbXBoYXNpemVzIHRoZSBpbXBvcnRhbmNlIG9mIGZsZXhpYmxlLCBjb21wcmVoZW5zaXZlIGFwcHJvYWNoZXMgdGhhdCBibGVuZCBzY2llbnRpZmljIGluc2lnaHRzIHdpdGggcHJhY3RpY2FsIHNvbHV0aW9ucyBhbmQgc3Ryb25nIHBvbGljeSBzdHJ1Y3R1cmVzIHRvIHRhY2tsZSB0aGlzIGludHJpY2F0ZSBlbnZpcm9ubWVudGFsIGNoYWxsZW5nZSBpbiBhbiBlcmEgb2YgY2xpbWF0ZSBjaGFuZ2UuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjIwOTQ0L3ByZXByaW50czIwMjQwOS4wNzA0LnYxIiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5wcmVwcmludHMub3JnL2Zyb250ZW5kL21hbnVzY3JpcHQvZTA4ZTViNTY1MzNlMmFhZDc0N2ZkOTQzNWU2N2NhNzMvZG93bmxvYWRfcHViIiwiaXNzdWVkRGF0ZSI6IjIwMjQtMDktMTAiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTEyIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJQcmVwcmludHMub3JnIiwiaW1wYWN0RmFjdG9yIjoiMS4xNSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MCwiY2l0ZWRCeUNvdW50Ijo1fX0sInF1b3RlIjoiVGhpcyByZXZpZXcgZXhhbWluZXMgZXV0cm9waGljYXRpb24gaW4gZnJlc2h3YXRlciBlY29zeXN0ZW1zLCBhIHdpZGVzcHJlYWQgZW52aXJvbm1lbnRhbCBpc3N1ZSBzdGVtbWluZyBmcm9tIGV4Y2Vzc2l2ZSBudXRyaWVudCBpbnB1dCwgbGFyZ2VseSBkdWUgdG8gaHVtYW4gYWN0aXZpdGllcy4gVGhlIHN0dWR5IGV4cGxvcmVzIHRoZSBvcmlnaW5zLCBjb25zZXF1ZW5jZXMsIGV2YWx1YXRpb24gdGVjaG5pcXVlcywgYW5kIGNvbnRyb2wgbWV0aG9kcyBmb3IgZXV0cm9waGljYXRpb24uIFByaW1hcnkgY2F1c2VzIGluY2x1ZGUgYWdyaWN1bHR1cmFsIGRpc2NoYXJnZSwgdXJiYW4gZGV2ZWxvcG1lbnQsIGFuZCBjbGltYXRlIHNoaWZ0cy4gVGhlIGVmZmVjdHMgYXJlIGZhci1yZWFjaGluZywgaW1wYWN0aW5nIGJpb2RpdmVyc2l0eSwgd2F0ZXIgcXVhbGl0eSwgYW5kIHNvY2lvLWVjb25vbWljIGZhY3RvcnMuIFRoZSBhc3Nlc3NtZW50IHV0aWxpemVzIGEgY29tYmluYXRpb24gb2YgY2hlbWljYWwsIHBoeXNpY2FsLCBhbmQgYmlvbG9naWNhbCBtYXJrZXJzLCB3aXRoIGN1dHRpbmctZWRnZSB0ZWNobm9sb2dpZXMgbGlrZSByZW1vdGUgc2Vuc2luZyIsImV4dGVybmFsQ2FjaGVJZCI6IjAyMzJlNjc0LTI0MjAtM2UzOS04ZDg2LTkwNzg4OWUwNWY4YiJ9XQ%3D%3D. The persistence of these toxic species is often facilitated by the dominance of cyanobacteria such as Dolichospermum and Aphanizomenon, which thrive in nutrient-rich, turbid conditions and outcompete beneficial phytoplankton (Jilbert et al., 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRvbSIsImZhbWlseSI6IkppbGJlcnQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUmFvdWzigJBNYXJpZSIsImZhbWlseSI6IkNvdXR1cmUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQnJpYW4iLCJmYW1pbHkiOiJIdXNlciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLYWxldmkiLCJmYW1pbHkiOiJTYWxvbmVuIn19XSwiaWRzIjp7Imlzc24iOlsiMDAxOC04MTU4IiwiMTU3My01MTE3Il0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDA3L3MxMDc1MC0wMjAtMDQ0NTcteCIsIm1hZyI6IjMxMDA0NTkwNTAiLCJvcGVuYWxleCI6IlczMTAwNDU5MDUwIiwib3BlbkFjY2Vzc0lkIjoiMTc2MjU4NzAwNTU2ODQ3NTQ0NTYifSwidGl0bGUiOiJQcmVmYWNlOiBSZXN0b3JhdGlvbiBvZiBldXRyb3BoaWMgbGFrZXM6IGN1cnJlbnQgcHJhY3RpY2VzIGFuZCBmdXR1cmUgY2hhbGxlbmdlcyIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDA3L3MxMDc1MC0wMjAtMDQ0NTcteCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9saW5rLnNwcmluZ2VyLmNvbS9jb250ZW50L3BkZi8xMC4xMDA3L3MxMDc1MC0wMjAtMDQ0NTcteC5wZGYiLCJpc3N1ZWREYXRlIjoiMjAyMC0xMS0xNiIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDciLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6Ikh5ZHJvYmlvbG9naWEiLCJpc3N1ZSI6MjEsInZvbHVtZSI6ODQ3LCJpbXBhY3RGYWN0b3IiOiIzLjAwIn0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6NDM0MywibGFzdFBhZ2UiOjQzNTd9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiU3ByaW5nZXIgU2NpZW5jZStCdXNpbmVzcyBNZWRpYSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6OTUsImNpdGVkQnlDb3VudCI6NTh9fSwicXVvdGUiOiJTaW1pbGFyIGRlZ3JhZGF0aW9uIG9mIGZyZXNod2F0ZXIgZWNvc3lzdGVtcyBpcyBvYnNlcnZlZCB3b3JsZHdpZGUsIHdpdGggdGhlIHRpbWVsaW5lIG9mIGh1bWFuIHByZXNzdXJlIGRpY3RhdGVkIGJ5IHRyZW5kcyBpbiB1cmJhbml6YXRpb24gYW5kIGludGVuc2l2ZSBhZ3JpY3VsdHVyZSBpbiBkaWZmZXJlbnQgZ2VvZ3JhcGhpYyByZWdpb25zIChlLmcuLCBWZW5uZW1vIGV0IGFsLiwgMjAwOSkuIFRoZSBwcmluY2lwYWwgZHJpdmVyIG9mIGVjb3N5c3RlbSBzdHJlc3MgaW4gbW9zdCBhcXVhdGljIHN5c3RlbXMgaXMgZXV0cm9waGljYXRpb24sIHRoZSBleGNlc3MgZ3Jvd3RoIG9mIGFsZ2FlIGRyaXZlbiBieSBhbnRocm9wb2dlbmljIGlucHV0cyBvZiB0aGUgbWFjcm9udXRyaWVudHMgbml0cm9nZW4gKE4pIGFuZCBwaG9zcGhvcnVzIChQKSAoU21pdGgsIDIwMDM7IEJpcmsgZXQgYWwuLCAyMDIwKS4gTnV0cmllbnRzIGFyZSBlbWl0dGVkIGZyb20gYm90aCBwb2ludCBhbmQgZGlmZnVzZSBzb3VyY2VzIGluIHRoZSBodW1hbiBlbnZpcm9ubWVudCBhbmQgZW50ZXIgYXF1YXRpYyBzeXN0ZW1zIHZpYSBzdXJmYWNlIHJ1bm9mZiwgYXMgd2VsbCBhcyBncm91bmQtd2F0ZXIgYW5kIGF0bW9zcGhlcmljIGRlcG9zaXRpb24gKFBhZXJsLCAxOTk3KS4gRXV0cm9waGljYXRpb24gb2Z0ZW4gbGVhZHMgdG8gdGhlIGRvbWluYW5jZSBvZiBwcmltYXJ5IHByb2R1Y3Rpb24gYnkgY3lhbm9iYWN0ZXJpYSwgaW5jbHVkaW5nIHRveGljIGdlbmVyYSBzdWNoIGFzIERvbGljaG9zcGVybXVtLCBBcGhhbml6b21lbm9uLCBhbmQgTWljcm9jeXN0aXMgKEh1aXNtYW4gZXQgYWwuLCAyMDE4KSwgYXMgd2VsbCBhcyBveHlnZW4gZGVwbGV0aW9uIGFuZCBoYWJpdGF0IGRlZ3JhZGF0aW9uIGluIGxha2VzIChBcmVuZCBldCBhbC4sIDIwMTEpLiBCZWNhdXNlIGxha2VzIHByb3ZpZGUgYSByYW5nZSBvZiBjcnVjaWFsIGVjb3N5c3RlbSBzZXJ2aWNlcyAoV2lsc29uICYgQ2FycGVudGVyLCAxOTk5KSwgdGhlcmUgaXMgYSBncm93aW5nIG5lZWQgZm9yIG1hbmFnZW1lbnQgYXBwcm9hY2hlcyB0aGF0IHJlc3RvcmUgd2F0ZXIgcXVhbGl0eSBpbiBmcmVzaHdhdGVyIHN5c3RlbXMuIFdpdGhpbiB0aGUgZnJhbWV3b3JrIG9mIFVuaXRlZCBOYXRpb25zIFN1c3RhaW5hYmxlIERldmVsb3AtbWVudCBHb2FsIDYgKENsZWFuIFdhdGVyIGFuZCBTYW5pdGF0aW9uKSwgc2V2ZXJhbCBkaXN0aW5jdCB0YXJnZXRzIHJlbGF0ZSB0byB0aGUgaW1wcm92ZW1lbnQgb2Ygd2F0ZXIgcXVhbGl0eSBhbmQgcHJvdGVjdGlvbiBvZiBhcXVhdGljIGVjb3N5c3RlbXMgYnkgMjAzMCAoVW5pdGVkIE5hdGlvbnMsIDIwMTUpLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI4LjI5ODg2NDM2NDYyNDAyMywibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNzE0OTQyOTMyMTI4OSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyOS45OTExMzQ2NDM1NTQ2ODgsImxlZnQiOjguNTQ5MjA1NzgwMDI5Mjk3LCJ3aWR0aCI6MzkuMzY1MTE5OTM0MDgyMDMsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzEuNjgzNDA2ODI5ODMzOTg0LCJsZWZ0Ijo4LjU0OTIwNTc4MDAyOTI5Nywid2lkdGgiOjM5LjM2OTY2MzIzODUyNTM5LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMzLjM2Nzk3MzMyNzYzNjcyLCJsZWZ0Ijo4LjU0OTIwNTc4MDAyOTI5Nywid2lkdGgiOjM5LjM3NTY3OTAxNjExMzI4LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjM1LjA2MDI0OTMyODYxMzI4LCJsZWZ0Ijo4LjU0OTIwNTc4MDAyOTI5Nywid2lkdGgiOjM5LjM3MDk2NDA1MDI5Mjk3LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjM2Ljc1MjUxNzcwMDE5NTMxLCJsZWZ0Ijo4LjU0OTIwODY0MTA1MjI0Niwid2lkdGgiOjM5LjM3Njk2MDc1NDM5NDUzLCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjM4LjQzNzA4ODAxMjY5NTMxLCJsZWZ0Ijo4LjU0OTIwODY0MTA1MjI0Niwid2lkdGgiOjM5LjM2NjU2OTUxOTA0Mjk3LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQwLjEyOTM1NjM4NDI3NzM0NCwibGVmdCI6OC41NDkyMDg2NDEwNTIyNDYsIndpZHRoIjozOS4zNjQzNzk4ODI4MTI1LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQxLjgyMTYyNDc1NTg1OTM3NSwibGVmdCI6OC41NDkyMDg2NDEwNTIyNDYsIndpZHRoIjozOS4zNjU2NzMwNjUxODU1NSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0My41MDYxOTUwNjgzNTkzNzUsImxlZnQiOjguNTQ5Mzg5ODM5MTcyMzYzLCJ3aWR0aCI6MzkuMzcwMzkxODQ1NzAzMTI1LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQ1LjE5ODQ2MzQzOTk0MTQwNiwibGVmdCI6OC41NDk1NzI5NDQ2NDExMTMsIndpZHRoIjozOS4zNjU4NDQ3MjY1NjI1LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQ2Ljg5MDczNTYyNjIyMDcsImxlZnQiOjguNTQ5NTcyOTQ0NjQxMTEzLCJ3aWR0aCI6MzkuMzcyOTU1MzIyMjY1NjI1LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQ4LjU3NTMwMjEyNDAyMzQ0LCJsZWZ0Ijo4LjU0OTU3Mjk0NDY0MTExMywid2lkdGgiOjM5LjM3MjQyMTI2NDY0ODQ0LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjUwLjI2NzU3ODEyNSwibGVmdCI6OC41NDk1NzY3NTkzMzgzNzksIndpZHRoIjozOS4zNzExMjQyNjc1NzgxMjUsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTEuOTU5ODQyNjgxODg0NzY2LCJsZWZ0Ijo4LjU0OTU3Njc1OTMzODM3OSwid2lkdGgiOjM5LjM2NzgzNTk5ODUzNTE1NiwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1My41NzQxMTk1Njc4NzEwOTQsImxlZnQiOjguNTQ5NTc2NzU5MzM4Mzc5LCJ3aWR0aCI6MzkuMzcwNzc3MTMwMTI2OTUsImhlaWdodCI6MS4zMzY4OTU1ODUwNjAxMTk2fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTUuMjY2MzkxNzU0MTUwMzksImxlZnQiOjguNTQ5NzYwODE4NDgxNDQ1LCJ3aWR0aCI6MzkuMzY2NTgwOTYzMTM0NzY2LCJoZWlnaHQiOjEuMzM2ODk1NTg1MDYwMTE5Nn19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjU3LjAyODk1MzU1MjI0NjA5NCwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNjIwMjYyMTQ1OTk2MSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1OC43MTM1MjM4NjQ3NDYwOTQsImxlZnQiOjguNTQ5OTQ0ODc3NjI0NTEyLCJ3aWR0aCI6OS4yOTk5ODIwNzA5MjI4NTIsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NjAuNDA1NzkyMjM2MzI4MTI1LCJsZWZ0IjoxMC42MjI0NTU1OTY5MjM4MjgsIndpZHRoIjozNy4yODc4Njg0OTk3NTU4NiwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2Mi4wOTgwNjA2MDc5MTAxNTYsImxlZnQiOjguNTQ5OTQzOTIzOTUwMTk1LCJ3aWR0aCI6MzkuMzczMzI1MzQ3OTAwMzksImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NjMuNzgyNjMwOTIwNDEwMTU2LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjM5LjM3MzEyNjk4MzY0MjU4LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjY1LjQ3NDkwNjkyMTM4NjcyLCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjM5LjM2NTExOTkzNDA4MjAzLCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjY3LjE2NzE3NTI5Mjk2ODc1LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjM5LjM2OTQ4MDEzMzA1NjY0LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjY4Ljc4MTQ0ODM2NDI1NzgxLCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjM5LjM3NzMzODQwOTQyMzgzLCJoZWlnaHQiOjEuMzM2ODkxNTMxOTQ0Mjc1fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NzAuNTQ0MDEzOTc3MDUwNzgsImxlZnQiOjguNTQ5OTQ0ODc3NjI0NTEyLCJ3aWR0aCI6MzkuMzY5MzA4NDcxNjc5NjksImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NzIuMjM2Mjg5OTc4MDI3MzQsImxlZnQiOjguNTQ5OTQ0ODc3NjI0NTEyLCJ3aWR0aCI6MzkuMzcyMDMyMTY1NTI3MzQ0LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjczLjkyMDg0NTAzMTczODI4LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjM5LjM3MTMwNzM3MzA0Njg3NSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fV19LCJjaHVua0lkIjoiMjJmZWI3YmEtYzU2NS00ZTM2LTgxZTMtOTM2ZTQxNzdlMmQxIiwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNDM0NCJ9fV0%3D. This competitive advantage is reinforced by their physiological adaptations, such as the ability to regulate buoyancy and utilize atmospheric nitrogen, which allows them to form dense surface scums that block light penetration and suppress submerged macrophytes (Çelekli̇, 2024; Shahbaz et al., 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFidXplciIsImZhbWlseSI6IsOHZWxla2xpzIcifX1dLCJpZHMiOnsiaXNzbiI6W10sImlzYm4iOltdLCJkb2kiOiIxMC4yMTIwMy9ycy4zLnJzLTUwNzgxNzYvdjEiLCJvcGVuYWxleCI6Ilc0NDA0NTI5NDE4Iiwib3BlbkFjY2Vzc0lkIjoiMzI4MDQ0NDAzMjU0NjQxMzgwMSJ9LCJ0aXRsZSI6IlRoZSBmaXJzdCBlY28tYXNzZXNzbWVudCBvZiBwaHl0b3BsYW5rdG9uIGFzc2VtYmxhZ2VzIGFzc29jaWF0ZWQgd2l0aCBlbnZpcm9ubWVudGFsIGNvbmRpdGlvbnMgb2YgZGlmZmVyZW50IGxha2VzIGluIHRoZSBXZXN0ZXJuIEJsYWNrIFNlYSBiYXNpbiBvZiBUw7xya2l5ZSIsImFic3RyYWN0IjoiPHRpdGxlPkFic3RyYWN0PC90aXRsZT4gTGFrZXMgcGxheSBhIHZpdGFsIHJvbGUgaW4gc3VwcG9ydGluZyBiaW9kaXZlcnNpdHksIHByb3ZpZGluZyB3YXRlciByZXNvdXJjZXMsIHJlZ3VsYXRpbmcgY2xpbWF0ZSwgY3ljbGluZyBudXRyaWVudHMsIGFuZCBvZmZlcmluZyByZWNyZWF0aW9uYWwgb3Bwb3J0dW5pdGllcy4gRGVzcGl0ZSB0aGVpciBpbXBvcnRhbmNlIGZvciBlbnZpcm9ubWVudGFsIGhlYWx0aCBhbmQgaHVtYW4gd2VsbC1iZWluZywgbGFrZXMgZmFjZSBzaWduaWZpY2FudCBwcmVzc3VyZXMgaW4gdGhlIEFudGhyb3BvY2VuZSBlcmEuIFRoZSBwcmVzZW50IHdvcmsgc2Vla3MgdG8gYXNzZXNzIHRoZSBzcGVjaWVzLWVudmlyb25tZW50IGludGVyYWN0aW9ucyBhbmQgdGhlIGVjb2xvZ2ljYWwgc3RhdHVzIG9mIHNpeCBsYWtlcyBpbiB0aGUgV2VzdGVybiBCbGFjayBTZWEgYmFzaW4gb2YgVMO8cmtpeWUgdXRpbGl6aW5nIHBoeXRvcGxhbmt0b24gbWV0cmljcyBkdXJpbmcgd2V0IGFuZCBkcnkgcGVyaW9kcy4gQ2Fub25pY2FsIGNvcnJlc3BvbmRlbmNlIGFuYWx5c2lzIHJldmVhbGVkIGEgc2lnbmlmaWNhbnQgY29ycmVsYXRpb24gZXF1YWwgdG8gOTguNSUgYmV0d2VlbiBwaHl0b3BsYW5rdG9uIHNwZWNpZXMgYW5kIGVudmlyb25tZW50YWwgc3RyZXNzb3JzIGR1cmluZyB0d28gaHlkcm9sb2dpY2FsIHBlcmlvZHMuIEVsZWN0cmljYWwgY29uZHVjdGl2aXR5IChFQyksIHBILCB0b3RhbCBvcmdhbmljIGNhcmJvbiAoVE9DKSwgYW5kIHRlbXBlcmF0dXJlIHdlcmUgdGhlIG1vc3QgaW5mbHVlbnRpYWwgZW52aXJvbm1lbnRhbCBmYWN0b3JzIGFmZmVjdGluZyBwaHl0b3BsYW5rdG9uIGRpc3RyaWJ1dGlvbiBpbiBsYWtlcyAocD0wLjAwMikuIExha2UgU2FyxLFrdW0sIGEgYnJhY2tpc2ggaGFiaXRhdCwgd2FzIHVuZGVyIGhpZ2ggRUMgYW5kIHBIIHByZXNzdXJlIGFuZCBpcyBjaGFyYWN0ZXJpemVkIGJ5IHBvbGx1dGlvbi10b2xlcmFudCBzcGVjaWVzLiBMYWtlIFllbmnDp2HEn2EgYXNzb2NpYXRlZCB3aXRoIFRPQyBpcyBjaGFyYWN0ZXJpemVkIGJ5IHNvbWUgc3BlY2llcywgc3VjaCBhcyA8aXRhbGljPkFuYWJhZW5vcHNpczwvaXRhbGljPiA8aXRhbGljPm1pbGxlcmk8L2l0YWxpYz4sIDxpdGFsaWM%2BQ2hyb29jb2NjdXM8L2l0YWxpYz4gPGl0YWxpYz50dXJnaWR1czwvaXRhbGljPiw8aXRhbGljPiBQc2V1ZG9zY2hyb2VkZXJpYTwvaXRhbGljPiA8aXRhbGljPnJvYnVzdGE8L2l0YWxpYz4sIDxpdGFsaWM%2BQXBoYW5vY2Fwc2E8L2l0YWxpYz4gc3AuLCA8aXRhbGljPk1lcmlzbW9wZWRpYTwvaXRhbGljPiA8aXRhbGljPmdsYXVjYTwvaXRhbGljPiwgPGl0YWxpYz5NaWNyYWN0aW5pdW08L2l0YWxpYz4gPGl0YWxpYz5xdWFkcmlzZXR1bTwvaXRhbGljPiwgYW5kPGl0YWxpYz4gTWljcm9jeXN0aXM8L2l0YWxpYz4gPGl0YWxpYz5hZXJ1Z2lub3NhPC9pdGFsaWM%2BLiBMYWtlIEFiYW50IGlzIGxvY2F0ZWQgb24gdGhlIG9wcG9zaXRlIHNpZGUgb2YgRUMsIFRPQywgYW5kIHRlbXBlcmF0dXJlLCB3aGljaCB3YXMgYXNzb2NpYXRlZCB3aXRoIHNvbWUgc3BlY2llcyBzdWNoIGFzIDxpdGFsaWM%2BQ3ltYmVsbGEgYWZmaW5pczwvaXRhbGljPiwgPGl0YWxpYz5BY2huYW50aGlkaXVtPC9pdGFsaWM%2BIDxpdGFsaWM%2BbWludXRpc3NpbXVtPC9pdGFsaWM%2BLCA8aXRhbGljPkVuY3lvbmVtYTwvaXRhbGljPiA8aXRhbGljPm1pbnV0dW08L2l0YWxpYz4sIDxpdGFsaWM%2BRS48L2l0YWxpYz4gPGl0YWxpYz5zaWxlc2lhY3VtPC9pdGFsaWM%2BLCBhbmQ8aXRhbGljPiBEaW5vYnJ5b248L2l0YWxpYz4gPGl0YWxpYz5kaXZlcmdlbnM8L2l0YWxpYz4uIFJlc3VsdHMgb2YgdGhlIHBoeXRvLWFzc2Vzc21lbnQgZGlzcGxheWVkIHRoYXQgdGhlIGVjb2xvZ2ljYWwgc3RhdHVzIChFUykgb2YgdGhlIHNhbXBsaW5nIHN0YXRpb25zIGR1cmluZyB0aGUgcmFpbnkgYW5kIGRyeSBwZXJpb2RzIHZhcmllZCBmcm9tIGJhZCB0byBoaWdoLiBUaGUgbW9kaWZpZWQgcGh5dG9wbGFua3RvbiB0cm9waGljIGluZGV4IChNUFRJKSBleGhpYml0ZWQgdGhhdCBhIG1vZGVyYXRlIEVTIHdhcyBmb3VuZCBpbiBMYWtlcyBTYXLEsWt1bSBhbmQgWWVuacOnYcSfYSwgd2hpbGUgb3RoZXJzIGhhZCBhIGdvb2QgRVMuIFRoZSBwcmVzZW50IHN0dWR5IGNvbu%2BsgXJtZWQgdGhhdCBwaHl0b3BsYW5rdG9uIGNvbW11bml0aWVzIGFyZSBzdHJvbmdseSBsaW5rZWQgdG8gdGhlIGVjb2xvZ2ljYWwgc3RhdHVzIG9mIGxha2VzIGluIHRoZSBXZXN0ZXJuIEJsYWNrIFNlYSBiYXNpbiwgd2hpY2ggY291bGQgYmUgYXNzZXNzZWQgdXNpbmcgdGhlIE1QVEkuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjIxMjAzL3JzLjMucnMtNTA3ODE3Ni92MSIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cucmVzZWFyY2hzcXVhcmUuY29tL2FydGljbGUvcnMtNTA3ODE3Ni9sYXRlc3QucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjQtMTEtMTkiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA4IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJSZXNlYXJjaCBTcXVhcmUgKFJlc2VhcmNoIFNxdWFyZSkifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiUmVzZWFyY2ggU3F1YXJlIChVbml0ZWQgU3RhdGVzKSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NTIsImNpdGVkQnlDb3VudCI6MH19LCJxdW90ZSI6IkRlc3BpdGUgdGhlaXIgZWNvbG9naWNhbCBzaWduae%2B%2FvWNhbmNlLCB0aGVzZSB3YXRlciBib2RpZXMgYXJlIGluY3JlYXNpbmdseSB0aHJlYXRlbmVkIGJ5IGFudGhyb3BvZ2VuaWMgYWN0aXZpdGllcyAoZS5nLiwgdXJiYW5pemF0aW9uLCBhZ3JpY3VsdHVyYWwgbGFuZCB1c2UsIHdhc3Rld2F0ZXIgZGlzY2hhcmdlLCByZWVkIGN1dHRpbmcsIHNhbmQgcmVtb3ZhbCwgZXRjLikgYW5kIGNsaW1hdGUgY2hhbmdlIChIw6RkZXIgZXQgYWwuIDIwMjA7IMOHZWxla2xpIGV0IGFsLiAyMDI0KS4gTnV0cmllbnQgZW5yaWNobWVudCBpbiBsZW50aWMgZWNvc3lzdGVtcyBpbXBhY3RzIG5vdCBvbmx5IHRoZSBwaHlzaWNvY2hlbWljYWwgZmFjdG9ycyBvZiBsYWtlcyBidXQgYWxzbyBuZWdhdGl2ZWx5IGFmZmVjdHMgYXF1YXRpYyBsaWZlLCBjYXVzaW5nIGhhcm1mdWwgYWxnYWwgYmxvb21zLCBleGNlc3NpdmUgZ3Jvd3RoIG9mIG51aXNhbmNlIG1hY3JvcGh5dGVzLCBhbmQgaW5jcmVhc2VkIO%2B%2FvXNoIG1vcnRhbGl0eSwgZXNwZWNpYWxseSBzZW5zaXRpdmUgc3BlY2llcyAoU3VyZXNoIGV0IGFsLiAyMDIzKS4gSXQgYWxzbyBkaXNydXB0cyB0aGUgaHlkcm9tb3JwaG9sb2d5IGFuZCBvdmVyYWxsIGZ1bmN0aW9uaW5nIG9mIHRoZXNlIGVjb3N5c3RlbXMuIERlc3BpdGUgZ2xvYmFsIHdhdGVyIHNob3J0YWdlcywgdGhlIGdyb3dpbmcgZGVncmFkYXRpb24gb2YgZWNvc3lzdGVtcyBhbmQgdGhlIG92ZXJ1c2Ugb2YgZnJlc2h3YXRlciByZXNvdXJjZXMgaGF2ZSBzaGlmdGVkIGF0dGVudGlvbiBiYWNrIHRvIGZyZXNod2F0ZXIgZWNvc3lzdGVtcyB0byBiZXR0ZXIgbWFuYWdlIGFuZCBwcmVzZXJ2ZSB3YXRlciByZXNvdXJjZXMgZ2xvYmFsbHkgKMOHZWxla2xpIGV0IGFsLiAyMDIxOyBTdXJlc2ggZXQgYWwuIDIwMjMpLiBBc3Nlc3NpbmcgdGhlIGVjb2xvZ2ljYWwgc3RhdHVzIG9mIGxha2VzIHRocm91Z2ggcGh5dG9wbGFua3RvbiBtZXRyaWNzIGlzIGEgY3J1Y2lhbCBjb21wb25lbnQgb2YgZW52aXJvbm1lbnRhbCBtb25pdG9yaW5nIGFuZCBtYW5hZ2VtZW50LCBhcyBpdCBhbGlnbnMgd2l0aCB0aGUgZ3VpZGVsaW5lcyBzZXQgYnkgdGhlIFdhdGVyIEZyYW1ld29yayBEaXJlY3RpdmUgKERpcmVjdGl2ZSAyMDAwKS4gUGh5dG9wbGFua3RvbiwgYXMgcHJpbWFyeSBwcm9kdWNlcnMsIGFyZSBzZW5zaXRpdmUgaW5kaWNhdG9ycyBvZiBjaGFuZ2VzIGluIHdhdGVyIHF1YWxpdHkgYW5kIGVjb3N5c3RlbSBoZWFsdGggKFJleW5vbGRzIGV0IGFsLiAyMDAyOyDDh2VsZWtsaSBldCBhbC4gMjAyM2EpLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjY2Ljk2NDIyNTc2OTA0Mjk3LCJsZWZ0Ijo0LjUzNDMxMzY3ODc0MTQ1NSwid2lkdGgiOjkwLjYxNDc2MTM1MjUzOTA2LCJoZWlnaHQiOjEuNzc1NTI5NjIzMDMxNjE2Mn19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjY5LjIzNjkwNzk1ODk4NDM4LCJsZWZ0Ijo0LjUzNDMxMzY3ODc0MTQ1NSwid2lkdGgiOjkwLjAxMDMwNzMxMjAxMTcyLCJoZWlnaHQiOjEuNzc1NTI5NjIzMDMxNjE2Mn19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjcxLjUwOTU5MDE0ODkyNTc4LCJsZWZ0Ijo0LjUzNDMxMzY3ODc0MTQ1NSwid2lkdGgiOjg1LjgxMjE3OTU2NTQyOTY5LCJoZWlnaHQiOjEuNzc1NTI5NjIzMDMxNjE2Mn19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjczLjc4MjI3MjMzODg2NzE5LCJsZWZ0Ijo0LjUzNDMxMzY3ODc0MTQ1NSwid2lkdGgiOjg2LjA3MDMxMjUsImhlaWdodCI6MS43NzU1Mjk2MjMwMzE2MTYyfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NzYuMDU0OTU0NTI4ODA4NiwibGVmdCI6NC41MzQzMTM2Nzg3NDE0NTUsIndpZHRoIjo4OS42MjM2NjQ4NTU5NTcwMywiaGVpZ2h0IjoxLjc3NTUyOTYyMzAzMTYxNjJ9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo3OC4zMjc2MjkwODkzNTU0NywibGVmdCI6NC41MzQzMTM2Nzg3NDE0NTUsIndpZHRoIjo4OS44NTk3NTY0Njk3MjY1NiwiaGVpZ2h0IjoxLjc3NTUyOTYyMzAzMTYxNjJ9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo4MC42MDAzMTg5MDg2OTE0LCJsZWZ0Ijo0LjUzNDMxMzY3ODc0MTQ1NSwid2lkdGgiOjg5LjY0MDgzODYyMzA0Njg4LCJoZWlnaHQiOjEuNzc1NTI5NjIzMDMxNjE2Mn19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjgyLjg3Mjk5MzQ2OTIzODI4LCJsZWZ0Ijo0LjUzNDMxMzY3ODc0MTQ1NSwid2lkdGgiOjkwLjgzNjYzOTQwNDI5Njg4LCJoZWlnaHQiOjQuMDQ4MTk0ODg1MjUzOTA2fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6ODcuNDE4MzQyNTkwMzMyMDMsImxlZnQiOjQuNTM0MzEzNjc4NzQxNDU1LCJ3aWR0aCI6MzMuNzkwOTM1NTE2MzU3NDIsImhlaWdodCI6MS43NzU1Mjk2MjMwMzE2MTYyfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6OTEuMjA2MTQ2MjQwMjM0MzgsImxlZnQiOjQuNTM0MzEzNjc4NzQxNDU1LCJ3aWR0aCI6ODQuNjc1Mzc2ODkyMDg5ODQsImhlaWdodCI6MS43NzU1Mjk2MjMwMzE2MTYyfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6OTMuNDc4ODIwODAwNzgxMjUsImxlZnQiOjQuNTM0MzEzNjc4NzQxNDU1LCJ3aWR0aCI6ODkuOTgzODQ4NTcxNzc3MzQsImhlaWdodCI6MS43NzU1Mjk2MjMwMzE2MTYyfX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6NC43NDk2NDM4MDI2NDI4MjIsImxlZnQiOjQuNTM0MzEzNjc4NzQxNDU1LCJ3aWR0aCI6ODcuOTU3ODYyODU0MDAzOSwiaGVpZ2h0IjoxLjc3NTUzMDU3NjcwNTkzMjZ9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo3LjAyMjMwOTMwMzI4MzY5MSwibGVmdCI6NC41MzQzMTM2Nzg3NDE0NTUsIndpZHRoIjo5MC42NjY1MTE1MzU2NDQ1MywiaGVpZ2h0IjoxLjc3NTUzMDU3NjcwNTkzMjZ9fV19LCJjaHVua0lkIjoiMzg5ZTRmM2EtZWRmMS00OTRhLWJiOWUtZWMyYWMxY2Q5MDJjIiwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiMiJ9fSx7Im9yaWdpbiI6NCwid29yayI6eyJ0eXBlIjo4LCJzdWJ0eXBlIjp7Im9uZW9mS2luZCI6ImFydGljbGVUeXBlIiwiYXJ0aWNsZVR5cGUiOjF9LCJjb250cmlidXRvcnMiOlt7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNZCIsImZhbWlseSI6IlNoYWhiYXoifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQWhiYXIiLCJmYW1pbHkiOiJBbGFtIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1vaGFtbWFkIE1hc3Jvb3IiLCJmYW1pbHkiOiJaYWZhciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNb2hhbW1lZCBBYXNpZiIsImZhbWlseSI6IlN1bGFpbWFuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFudXBtYSIsImZhbWlseSI6Ikt1bWFyaSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJHLiBLLiIsImZhbWlseSI6IlNoYXJtYSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTaGFobGEiLCJmYW1pbHkiOiJZYXNtaW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIxMjE1LTE2MTAiLCIyMDYxLTk1ODgiXSwiaXNibiI6W10sImRvaSI6IjEwLjI0Nzgvb3JodS0yMDIzLTAwMTYiLCJvcGVuYWxleCI6Ilc0Mzg5NTEwMDM3Iiwib3BlbkFjY2Vzc0lkIjoiMTY2NzQ1NjMwNzY4ODEzOTgyNzAifSwidGl0bGUiOiJDb21wYXJpc29uIG9mIGF2aWFuIGRpdmVyc2l0eSBiZXR3ZWVuIG1hbmFnZWQgYW5kIHVubWFuYWdlZCB3ZXRsYW5kcyBpbiBQYXRuYSwgQmloYXIsIEluZGlhIiwiYWJzdHJhY3QiOiJBYnN0cmFjdCBXZSBjb21wYXJlZCB0aGUgYmlyZCBkaXZlcnNpdHkgb2YgYSBtYW5hZ2VkIChSYWpkaGFuaSBKYWxhc2hheSkgYW5kIGFuIHVubWFuYWdlZCB3ZXRsYW5kIChNdXN0YWZhcHVyKSBpbiBQYXRuYSwgQmloYXIsIEluZGlhLiBXZSByZWNvcmRlZCBiaXJkIHNwZWNpZXMgYnkgdXNpbmcgcG9pbnQgY291bnRzIGFuZCBmaXhlZC1yb3V0ZSBtb25pdG9yaW5nLiBBdCBSYWpkaGFuaSBKYWxhc2hheSwgYSB0b3RhbCBvZiA3MyBzcGVjaWVzIHdlcmUgcmVjb3JkZWQsIG9mIHdoaWNoIDE1IHNwZWNpZXMgd2VyZSB3aW50ZXIgdmlzaXRvcnMuIFR3byBzcGVjaWVzIHdlcmUgaW4gdGhlIE5lYXItdGhyZWF0ZW5lZCBjYXRlZ29yeTogRmVycnVnaW5vdXMgRHVjayAoQXl0aHlhIG55cm9jYSkgYW5kIEFsZXhhbmRyaW5lIFBhcmFrZWV0IChQc2l0dGFjdWxhIGV1cGF0cmlhKSAsIHdoaWxlIHRoZSBvdGhlcnMgd2VyZSBvZiBsZWFzdCBjb25jZXJuLiBBdCBNdXN0YWZhcHVyIHdldGxhbmQsIDY3IHNwZWNpZXMgd2VyZSByZWNvcmRlZCB3aXRoIDExIHNwZWNpZXMgYXMgd2ludGVyIHZpc2l0b3JzLiBTaW1wc29u4oCZcyBzcGVjaWVzIGRpdmVyc2l0eSBpbmRleCB3YXMgMC43OCBhdCBSYWpkaGFuaSBKYWxhc2hheSwgYW5kIDAuODEgYXQgTXVzdGFmYXB1ciB3ZXRsYW5kLiBUaGUgbnVtYmVyIG9mIHNwZWNpZXMgb2Ygd2FkZXJzIHdhcyAxNyBhdCBNdXN0YWZhcHVyIHdldGxhbmQgYW5kIDExIGF0IFJhamRoYW5pIEphbGFzaGF5LiBUaGUgdG90YWwgbnVtYmVyIG9mIExlc3NlciBXaGlzdGxpbmctZHVjayAoRGVuZHJvY3lnbmEgamF2YW5pY2EpICwgYSByZXNpZGVudCBiaXJkIHdhcyBhbG1vc3QgZG91YmxlICg1MjYpIGF0IE11c3RhZmFwdXIgd2V0bGFuZCB0aGFuIHRoYXQgYXQgUmFqZGhhbmkgSmFsYXNoYXkgKDIzNCkuIFRoZSBuaXRyYXRlIGxldmVsIHdhcyB0d2ljZSBoaWdoZXIgYW5kIHRoZSBwaG9zcGhhdGUgbGV2ZWwgd2FzIGZvdXIgdGltZXMgaGlnaGVyIGF0IE11c3RhZmFwdXIgd2V0bGFuZCB0aGFuIHRob3NlIGF0IFJhamRoYW5pIEphbGFzaGF5LiBUaGUgZGlmZmVyZW5jZSBpbiBzcGVjaWVzIG51bWJlciBhbmQgY29tcG9zaXRpb24gYmV0d2VlbiB0aGUgdHdvIHdldGxhbmRzIG1heSBiZSBhdHRyaWJ1dGVkIHRvIHRoZSBldXRyb3BoaWNhdGlvbiByZXN1bHRpbmcgZnJvbSBoaWdoIG5pdHJhdGUgYW5kIHBob3NwaGF0ZSBsZXZlbHMgYXQgTXVzdGFmYXB1ciB3ZXRsYW5kLiBVbmZvcnR1bmF0ZWx5LCB0aGUgTXVzdGFmYXB1ciB3ZXRsYW5kIGlzIHVuZGVyIHRocmVhdCBkdWUgdG8gaHVtYW4gaW5mbHVlbmNlcy4gVGhlIHByb3RlY3Rpb24gYW5kIHN1c3RhaW5hYmxlIG1hbmFnZW1lbnQgb2YgbmF0dXJhbCB3ZXRsYW5kcyBpcyByZXF1aXJlZCBmb3Igc2F2aW5nIHRoZSBiaW9kaXZlcnNpdHkgb2YgdGhlIGFyZWEuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjI0Nzgvb3JodS0yMDIzLTAwMTYiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vc2NpZW5kby5jb20vcGRmLzEwLjI0Nzgvb3JodS0yMDIzLTAwMTYiLCJpc3N1ZWREYXRlIjoiMjAyMy0xMi0wMSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6Ik9ybmlzIEh1bmdhcmljYSIsImlzc3VlIjoyLCJ2b2x1bWUiOjMxLCJpbXBhY3RGYWN0b3IiOiIwLjkyIn0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTMsImxhc3RQYWdlIjoyOH19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJIdW5nYXJpYW4gT3JuaXRob2xvZ2ljYWwgYW5kIE5hdHVyZSBDb25zZXJ2YXRpb24gU29jaWV0eSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MjcsImNpdGVkQnlDb3VudCI6MX19LCJxdW90ZSI6Ikhvd2V2ZXIsIGhpZ2ggbGV2ZWxzIG9mIGV1dHJvcGhpY2F0aW9uIG1heSBhbHNvIHRyaWdnZXIgdGhlIHByb2xpZmVyYXRpb24gb2YgZ3JlZW4gYWxnYWUgYW5kICBkZWNsaW5lIG9mIG1hY3JvcGh5dGVzIGxlYWRpbmcgdG8gYSBoaWdoLXR1cmJpZGl0eSBzdGF0ZSBvZiB3YXRlciwgaW4gd2hpY2ggZmV3IHNwZWNpZXMgY2FuICBzdXJ2aXZlIChTY2hlZmZlciBldCBhbC4gMTk5MykuIFRvIGFkZHJlc3MgdGhlIGV1dHJvcGhpY2F0aW9uIGFuZCBleGNlc3MgbnV0cmllbnQgaXNzdWVzIGluICBhcXVhdGljIGVjb3N5c3RlbXMsIHBoeXRvcmVtZWRpYXRpb24gb2ZmZXJzIGEgcHJvbWlzaW5nIHNvbHV0aW9uIChMaXUgZXQgYWwuIDIwMjEpLiBBcXVhdGljICBwbGFudHMgbGlrZSBkdWNrd2VlZHMsIHdhdGVyIGxldHR1Y2UgKFBpc3RpYSBzdHJhdGlvdGVzKSwgYW5kIHdhdGVyIGxpbGllcyBoYXZlIGRlbW9uc3RyYXRlZCAgdGhlaXIgYWJpbGl0eSB0byBjb21iYXQgZXV0cm9waGljYXRpb24gYnkgYWJzb3JiaW5nIGFuZCBhc3NpbWlsYXRpbmcgZXhjZXNzIHBob3NwaG9ydXMgYW5kICBuaXRyb2dlbiAoTmFoYXIgJiBIb3F1ZSAyMDIxKS4gSG93ZXZlciwgdGhlc2UgcGxhbnRzIGNhbiBzb21ldGltZXMgZ3JvdyB1bmNvbnRyb2xsYWJseSwgIHBvdGVudGlhbGx5IGNvdmVyaW5nIHRoZSB3YXRlciBzdXJmYWNlcy4gVG8gYXZvaWQgc3VjaCBzY2VuYXJpb3MsIHJlZ3VsYXIgbW9uaXRvcmluZyBpcyBlc3NlbnRpYWwgKExpdSBldCBhbC4gMjAyMSkuIFByb21vdGluZyBwdWJsaWMgYXdhcmVuZXNzIGlzIHRoZSBjb3JuZXJzdG9uZSBmb3IgbW9iaWxpemluZyBzdXBwb3J0IGFuZCBlbnN1cmluZyBlZmZlY3RpdmUgZXV0cm9waGljYXRpb24gbWFuYWdlbWVudC4gQ29udHJvbGxlZCBlcmFkaWNhdGlvbiBvZiBleGNlc3MgIGZsb3JhIHdoZW4gbmVjZXNzYXJ5IGlzIHZpdGFsIHRvIHByZXZlbnQgb3Zlcmdyb3d0aC4gVGhpcyBpbnRlZ3JhdGVkIGFwcHJvYWNoIGNhbiBlZmZlY3RpdmVseSAgcmVkdWNlIGV1dHJvcGhpY2F0aW9uIGluIHRoZSB3YXRlciBib2RpZXMgKFdhbmcgZXQgYWwuIDIwMjIpLiBFYWNoIHNwZWNpZXMgb2Ygd2V0bGFuZC1kZXBlbmRlbnQgYmlyZCBoYXMgYSB1bmlxdWUgYW5kIGNvbXBsZXggc2V0IG9mIG5lZWRzIGZvciB3ZXRsYW5kIGhhYml0YXRzIHRoYXQgbWFrZXMgaXQgZGlmZmljdWx0IHRvIGdlbmVyYWxpemUgYWJvdXQgaG93IGxvc3Mgb3IgZGVncmFkYXRpb24gb2Ygd2V0bGFuZHMgYWZmZWN0cyBiaXJkIHBvcHVsYXRpb25zLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo3OS4yOTU2MzkwMzgwODU5NCwibGVmdCI6MTAuMzAzMDA2MTcyMTgwMTc2LCJ3aWR0aCI6NzkuOTUyNzI4MjcxNDg0MzgsImhlaWdodCI6MS43MDkxOTc4Nzg4Mzc1ODU0fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjgxLjIxMjUzOTY3Mjg1MTU2LCJsZWZ0IjoxMC4zMDMwMDYxNzIxODAxNzYsIndpZHRoIjo3OS45NTkyODE5MjEzODY3MiwiaGVpZ2h0IjoxLjcwOTE5Nzg3ODgzNzU4NTR9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6ODMuMTI5NDQ3OTM3MDExNzIsImxlZnQiOjEwLjMwMzAwNjE3MjE4MDE3Niwid2lkdGgiOjc5Ljk0MjcyNjEzNTI1MzksImhlaWdodCI6MS43MDkxOTc4Nzg4Mzc1ODU0fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjg1LjA0NjM0ODU3MTc3NzM0LCJsZWZ0IjoxMC4zMDMwMDgwNzk1Mjg4MDksIndpZHRoIjo3OS45Mzk0NTMxMjUsImhlaWdodCI6MS43MDkxOTc4Nzg4Mzc1ODU0fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjg2Ljk2MzI1NjgzNTkzNzUsImxlZnQiOjEwLjMwMzAxMDk0MDU1MTc1OCwid2lkdGgiOjc5Ljk0MTg0ODc1NDg4MjgxLCJoZWlnaHQiOjEuNzA5MTk3ODc4ODM3NTg1NH19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo4OC44ODAxNjUxMDAwOTc2NiwibGVmdCI6MTAuMzAzMDEzODAxNTc0NzA3LCJ3aWR0aCI6NzkuOTM3NSwiaGVpZ2h0IjoxLjcwOTE5Nzg3ODgzNzU4NTR9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6OTAuNzk3MDY1NzM0ODYzMjgsImxlZnQiOjEwLjMwMzAxMzgwMTU3NDcwNywid2lkdGgiOjc5LjkzOTY1MTQ4OTI1NzgxLCJoZWlnaHQiOjEuNzA5MTk3ODc4ODM3NTg1NH19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo5Mi43MTM5NzM5OTkwMjM0NCwibGVmdCI6MTAuMzAzMDEzODAxNTc0NzA3LCJ3aWR0aCI6NzkuOTUwMzI1MDEyMjA3MDMsImhlaWdodCI6MS43MDkxOTc4Nzg4Mzc1ODU0fX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjEyLjIwNzk5MDY0NjM2MjMwNSwibGVmdCI6MTAuMzAzMDI0MjkxOTkyMTg4LCJ3aWR0aCI6NzkuOTUyNTE0NjQ4NDM3NSwiaGVpZ2h0Ijo3LjQ1OTkyMjc5MDUyNzM0NH19LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjoxOS44NzU2MjU2MTAzNTE1NjIsImxlZnQiOjEyLjEyMTMyNDUzOTE4NDU3LCJ3aWR0aCI6NzguMTE5MjI0NTQ4MzM5ODQsImhlaWdodCI6MS43MDkxOTc4Nzg4Mzc1ODU0fX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjIxLjc5MjUzMzg3NDUxMTcyLCJsZWZ0IjoxMC4zMDMwMjkwNjAzNjM3Nywid2lkdGgiOjc5Ljk0OTI0OTI2NzU3ODEyLCJoZWlnaHQiOjEuNzA5MTk3ODc4ODM3NTg1NH19LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjoyMy43MDk0MzY0MTY2MjU5NzcsImxlZnQiOjEwLjMwMzAyOTA2MDM2Mzc3LCJ3aWR0aCI6NzkuOTUyMjcwNTA3ODEyNSwiaGVpZ2h0IjoxLjcwOTE5Nzg3ODgzNzU4NTR9fV19LCJjaHVua0lkIjoiM2RhZmM0ZWEtODI4NC00MDM4LWI2N2ItYTQ4YWNmNTRhMGFjIiwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiMjUifX1d.

3. The Primary Driver: Eutrophication and Nutrient Overloading

Eutrophication represents the primary driver of harmful algal blooms in India's lakes, characterized by excessive nutrient enrichment primarily nitrogen (N) and phosphorus (P) from anthropogenic sources such as agricultural runoff, untreated sewage, and industrial effluents (Kumar & Mahajan, 2020; Nair & Nayak, 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlBhd2FuIiwiZmFtaWx5IjoiS3VtYXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW1icmlzaCBLdW1hciIsImZhbWlseSI6Ik1haGFqYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyNTIzLTM5NjMiLCIyNTIzLTM5NzEiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMDcvczQyNDUyLTAyMC0zMDgyLTgiLCJtYWciOiIzMDM1OTYwMjI4Iiwib3BlbmFsZXgiOiJXMzAzNTk2MDIyOCJ9LCJ0aXRsZSI6IlRyb3BoaWMgc3RhdHVzIGFuZCBpdHMgcmVndWxhdGluZyBmYWN0b3IgZGV0ZXJtaW5hdGlvbiBhdCB0aGUgUmV3YWxzYXIgTGFrZSwgbm9ydGh3ZXN0IEhpbWFsYXlhIChIUCksIEluZGlhLCBiYXNlZCBvbiBzZWxlY3RlZCBwYXJhbWV0ZXJzIGFuZCBtdWx0aXZhcmlhdGUgc3RhdGlzdGljYWwgYW5hbHlzaXMiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAwNy9zNDI0NTItMDIwLTMwODItOCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9saW5rLnNwcmluZ2VyLmNvbS9jb250ZW50L3BkZi8xMC4xMDA3L3M0MjQ1Mi0wMjAtMzA4Mi04LnBkZiIsImlzc3VlZERhdGUiOiIyMDIwLTA2LTIyIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiU04gQXBwbGllZCBTY2llbmNlcyIsImlzc3VlIjo3LCJ2b2x1bWUiOjIsImltcGFjdEZhY3RvciI6IjcuMzUifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiU3ByaW5nZXIgTmF0dXJlIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo0MCwiY2l0ZWRCeUNvdW50IjoxOX19LCJxdW90ZSI6IkNoYW5naW5nIHF1YWxpdHkgaGFzIGNyZWF0ZWQgdGhlIHdhdGVyIHNjYXJjaXR5IGFuZCAgYSBodWdlIG1hc3Mgb2YgcG9wdWxhdGlvbiB0aHVzIGJlZW4gZGVwcml2ZWQgZnJvbSB1c2luZyAgdGhlIGZyZXNod2F0ZXIgZm9yIHZhcmlvdXMgcHVycG9zZXMgWzMsIDRdLiBIb3dldmVyLCB0aGUgIG9ibm94aW91cyBtYXRlcmlhbHMgdGhhdCBzdGF5IGluIHdhdGVyIGJvZGllcyBmb3IgYSBsb25nLXRpbWUgcGVyaW9kIGJyaW5nIGNoYW5nZXMgaW4gdGhlIHBoeXNpb2NoZW1pY2FsIGNoYXItYWN0ZXJpc3RpY3MgYXMgd2VsbCBhcyB0aGUgbmF0dXJhbCBzY2VuYXJpbyBvZiB0aGUgbGFrZXMgWzVdLiAgVGhlIEhpbWFsYXlhbiBsYWtlcyBhcmUgdW5kZXIgc3RyZXNzIGR1ZSB0byBtaXNtYW5hZ2VkIGh1bWFuIGFjdGl2aXRpZXMsIHdoaWNoIGluY3JlYXNlIG51dHJpZW50cyBhbmQgcG9sbHV0aW9uIGluIHRlcm1zIG9mIGhlYXZ5IG1ldGFscyB0b3hpY2l0eSBhbmQgYmFjdGVyaWFsIGNvbnRhbWluYXRpb24gIFs24oCTOF0uIFRoZSBtYW5hZ2VtZW50IGFuZCBwcmVzZXJ2YXRpb24gb2YgdGhlIGxha2VzIGF0ICBwcmVzZW50IGhhdmUgYmVjb21lIGEgZ3JlYXQgY2hhbGxlbmdlIGZvciBlbnZpcm9ubWVudGFsLWlzdHMgYW5kIG1hbmFnZXJzIFs1LCA5XS4gVGhlIGV1dHJvcGhpY2F0aW9uIGlzIGFsc28gYSBtYWluIHByb2JsZW0gb2YgdGhlICBmcmVzaHdhdGVyIGxha2VzLCBhbmQgaXQgaXMgc3BlY2lmaWVkIGZvciB0aGUgbGFrZXMgaGF2aW5nICBudXRyaWVudCBlbnJpY2htZW50LiBJdCBjYXVzZXMgZXhjZXNzaXZlIGdyb3d0aCBvZiBhbGdhZSAgYW5kIGlzIGNvbnNpZGVyZWQgdG8gYmUgcmVzcG9uc2libGUgZm9yIGRlZ3JhZGluZyB0aGUgIHF1YWxpdGF0aXZlIG5hdHVyZSBvZiB0aGUgbGFrZXMgWzUsIDEwXS4gVGhlIGV1dHJvcGhpY2F0aW9uICBpcyBmb2xsb3dlZCBieSB0aGUgYWxnYWwgYmxvb20sIHdoaWNoIGdlbmVyYXRlcyB0b3hpbiwgIHdvcnNlbnMgd2F0ZXIgcXVhbGl0eSwgZGVwbGV0ZXMgZGlzc29sdmVkIG94eWdlbiAoRE8pICogUGF3YW4gS3VtYXIsIHBhd2FuLmV2c0BnbWFpbC5jb20gfCAxRGVwYXJ0bWVudCBvZiBFbnZpcm9ubWVudGFsIFNjaWVuY2VzLCBDZW50cmFsIFVuaXZlcnNpdHkgb2YgSGltYWNoYWwgUHJhZGVzaCwgRGhhcmFtc2hhbGEgMTc2MjE1LCBJbmRpYS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2OC41Nzg0NjA2OTMzNTkzOCwibGVmdCI6OC41NzE0MTg3NjIyMDcwMzEsIndpZHRoIjo0MC4zMjc1MTQ2NDg0Mzc1LCJoZWlnaHQiOjEuNTE5ODU1MjYwODQ4OTk5fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NzAuMTU4OTk2NTgyMDMxMjUsImxlZnQiOjguNTY5NzM4Mzg4MDYxNTIzLCJ3aWR0aCI6NDAuMzU0NzI0ODg0MDMzMiwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjcxLjczOTU0NzcyOTQ5MjE5LCJsZWZ0Ijo4LjU2OTczODM4ODA2MTUyMywid2lkdGgiOjQwLjM0OTY5MzI5ODMzOTg0NCwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjczLjMyMDA5MTI0NzU1ODYsImxlZnQiOjguNTY4MDU5OTIxMjY0NjQ4LCJ3aWR0aCI6NDAuMDIwMDg4MTk1ODAwNzgsImhlaWdodCI6MS41MTk4NTUyNjA4NDg5OTl9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo3NC45MDA2NDIzOTUwMTk1MywibGVmdCI6OC41NjgwNTk5MjEyNjQ2NDgsIndpZHRoIjozOS45OTk5Mjc1MjA3NTE5NSwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjc2LjQ4MTE4NTkxMzA4NTk0LCJsZWZ0Ijo4LjU2ODA1NzA2MDI0MTcsIndpZHRoIjo0MC4zNTcwNzQ3Mzc1NDg4MywiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjc4LjA2MTcyOTQzMTE1MjM0LCJsZWZ0Ijo4LjU2OTczNjQ4MDcxMjg5LCJ3aWR0aCI6NDAuMzUzNzEzOTg5MjU3ODEsImhlaWdodCI6MS41MTk4NTUyNjA4NDg5OTl9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2MC42NzU3Mjc4NDQyMzgyOCwibGVmdCI6NTEuNDI3MTY1OTg1MTA3NDIsIndpZHRoIjo0MC4zNzg1NjY3NDE5NDMzNiwiaGVpZ2h0IjoxOC45MDU4NTUxNzg4MzMwMDh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo4MS45MjczMzc2NDY0ODQzOCwibGVmdCI6OC40MDE0OTc4NDA4ODEzNDgsIndpZHRoIjo3My45NTQ2OTY2NTUyNzM0NCwiaGVpZ2h0IjoyLjY3ODc2MjY3NDMzMTY2NX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEifX0seyJvcmlnaW4iOjQsIndvcmsiOnsidHlwZSI6OCwic3VidHlwZSI6eyJvbmVvZktpbmQiOiJhcnRpY2xlVHlwZSIsImFydGljbGVUeXBlIjoxfSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTmF2eWEgVmlrcmFtYW4iLCJmYW1pbHkiOiJOYWlyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlByYXRlZXAgS3VtYXIiLCJmYW1pbHkiOiJOYXlhayJ9fV0sImlkcyI6eyJpc3NuIjpbIjIyOTYtNzc0NSJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMzM4OS9mbWFycy4yMDIzLjEwODcyOTYiLCJvcGVuYWxleCI6Ilc0MzgyNzIzMTMzIn0sInRpdGxlIjoiVW5jb3ZlcmluZyB3YXRlciBxdWFsaXR5IGFuZCBldmFsdWF0aW5nIHZ1bG5lcmFiaWxpdGllcyBvZiBzbWFsbC1zY2FsZSBmaXNoZXJpZXMgaW4gQ2hpbGlrYSBMYWdvb24sIEluZGlhIiwiYWJzdHJhY3QiOiJTbWFsbC1zY2FsZSBmaXNoZXJpZXMgKFNTRnMpIHBsYXkgYSBzdHJvbmcgcm9sZSBpbiBzdXN0YWluaW5nIG1pbGxpb25zIG9mIGxpdmVsaWhvb2RzLCBmb29kIHNlY3VyaXR5LCBudXRyaXRpb24sIGFuZCBpbmNvbWUgZ2xvYmFsbHkgYnV0IHRoZSBmaXNoZXJzIGVuZ2FnZWQgaW4gdGhpcyBzZWN0b3Igc2ltdWx0YW5lb3VzbHkgZXhwZXJpZW5jZSBoaWdoIGxldmVscyBvZiB2dWxuZXJhYmlsaXR5IGFuZCBwcm9jZXNzZXMgb2YgbWFyZ2luYWxpc2F0aW9uLiBTZXZlcmFsIGZhY3RvcnMgYXJlIGF0dHJpYnV0ZWQgdG8gdGhlIG11bHRpZGltZW5zaW9uYWwgdnVsbmVyYWJpbGl0aWVzIHRoZSBzbWFsbC1zY2FsZSBmaXNoZXJzIGV4cGVyaWVuY2UsIHNwYW5uaW5nIGJvdGggbmF0dXJhbCAoZS5nLiwgbmF0dXJhbCBkaXNhc3RlcnMsIGVjb3N5c3RlbSBjaGFuZ2UpIGFuZCBhbnRocm9wb2dlbmljIChlLmcuLCBwb2xpY3kgY2hhbmdlLCBoeWRyb2xvZ2ljYWwgaW50ZXJ2ZW50aW9ucywgYXF1YWN1bHR1cmUpIHByZXNzdXJlcy4gV2hpbGUgdGhlcmUgaXMgbXVjaCBsaXRlcmF0dXJlIG9uIHZhcmlvdXMgbmF0dXJhbCBhbmQgaHVtYW4gZHJpdmVycyBvZiB2dWxuZXJhYmlsaXR5IGluIHNtYWxsLXNjYWxlIGZpc2hlcnkgY29tbXVuaXRpZXMsIGFuIGFic2VuY2Ugb2YgcmVzZWFyY2ggY29ubmVjdGluZyB2dWxuZXJhYmlsaXR5IHdpdGggd2F0ZXIgcXVhbGl0eSBpcyBldmlkZW50LiBGaXNoZXIgY29tbXVuaXRpZXMgb2Z0ZW4gdGFsayBhYm91dCBmaXNoIGluIHJlbGF0aW9uIHRvIHRoZSBoZWFsdGggb2YgdGhlaXIgYXF1YXRpYyBoYWJpdGF0cyB3aGVyZWluIHdhdGVyIHF1YWxpdHkgaXMgc2VlbiBhcyBhIGtleSBwYXJhbWV0ZXIuIFRoZSBsaW5rIGJldHdlZW4gaGVhbHRoeSBmaXNoIGFuZCBnb29kIHF1YWxpdHkgd2F0ZXIgaGFzIHNpZ25pZmljYW50IGltcGxpY2F0aW9ucyBmb3Igc3Ryb25nIGFuZCB2aWFibGUgZmlzaGluZyBjb21tdW5pdGllcy4gVGhpcyBwYXBlciBleGFtaW5lcyB0aGVzZSBsaW5rcyBmdXJ0aGVyIGJ5IGZvY3VzaW5nIG9uIHRoZSBuYXR1cmUgb2YgdnVsbmVyYWJpbGl0aWVzIGNhdXNlZCBieSB3YXRlciBxdWFsaXR5IGNoYW5nZXMgaW4gdGhlIHNtYWxsLXNjYWxlIGZpc2hlcnkgc3lzdGVtIG9mIENoaWxpa2EgTGFnb29uIGluIEluZGlhLiBXZSB1bmRlcnRha2UgZGV0YWlsZWQgYW5hbHlzaXMgb2YgdGhlIGludmFzaXZlIHNocmltcCBhcXVhY3VsdHVyZSBhY3Rpdml0aWVzIGFuZCBoeWRyb2xvZ2ljYWwgaW50ZXJ2ZW50aW9ucyBmb3Igb3BlbmluZyBvZiBhIGxhZ29vbiBpbmxldCB3aXRoIHRoZSBCYXkgb2YgQmVuZ2FsIGFzIHR3byBkb21pbmFudCBkcml2ZXJzIGFkdmVyc2VseSBpbXBhY3Rpbmcgd2F0ZXIgcXVhbGl0eSBhbmQgaW5jcmVhc2luZyB2dWxuZXJhYmlsaXRpZXMgb2YgdGhlIGVudGlyZSBzbWFsbC1zY2FsZSBmaXNoZXJpZXMgc29jaWFsLWVjb2xvZ2ljYWwgc3lzdGVtLiBPdXIgYW5hbHlzaXMgc3VnZ2VzdHMgdGhhdCB0aGVyZSBhcmUgc3Ryb25nIGludGVyY29ubmVjdGlvbnMgYmV0d2VlbiBjaGFuZ2VzIGluIHdhdGVyIHF1YWxpdHkgYW5kIHRoZSBsZXZlbHMgb2YgdnVsbmVyYWJpbGl0aWVzIGluIHRoZSBTU0ZzIG9mIENoaWxpa2EgTGFnb29uLiBQb2xsdXRhbnRzIHN1Y2ggYXMgcGVzdGljaWRlcywgYW5kIG9yZ2FuaWMgY29tcG91bmRzIGFjY3VtdWxhdGUgaW4gZmlzaCB0aXNzdWVzIGFuZCBhZmZlY3QgdGhlaXIgZ3Jvd3RoLCByZXByb2R1Y3Rpb24sIGFuZCBvdmVyYWxsIGhlYWx0aC4gVGhpcyBsZWQgdG8gZGVjbGluZXMgaW4gZmlzaCBwb3B1bGF0aW9ucywgbWFraW5nIGl0IG1vcmUgZGlmZmljdWx0IGZvciBmaXNoZXJzIHRvIG1ha2UgYSBsaXZpbmcuIEluIGFkZGl0aW9uIHRvIGRpcmVjdCBpbXBhY3RzIG9uIGZpc2ggcG9wdWxhdGlvbnMsIHBvb3Igd2F0ZXIgcXVhbGl0eSBhbHNvIGhhcyBpbmRpcmVjdCBlZmZlY3RzIG9uIHRoZSBzb2NpYWwgYW5kIGVjb25vbWljIHZ1bG5lcmFiaWxpdHkgb2YgU1NGcy4gRm9yIGV4YW1wbGUsIGNvbnRhbWluYXRpb24gb2Ygd2F0ZXIgc291cmNlcyBsZWQgdG8gdGhlIHJlZHVjZWQgbnVtYmVyIG9mIGZpc2ggc3BlY2llcyByZWR1Y2luZyB0aGUgYW1vdW50IG9mIHRpbWUgZmlzaGVycyBjYW4gc3BlbmQgb24gZmlzaGluZyBhY3Rpdml0aWVzLiBUaGlzIGFsc28gYWZmZWN0ZWQgdGhlIG1hcmtldGFiaWxpdHkgb2YgZmlzaCBwcm9kdWN0cywgcmVkdWNlZCBpbmNvbWUgYW5kIGluY3JlYXNlZCBwb3ZlcnR5LiBUbyBmdWxseSB1bmRlcnN0YW5kIHRoZSBpbnRlcmNvbm5lY3Rpb25zIGJldHdlZW4gd2F0ZXIgcXVhbGl0eSBhbmQgdnVsbmVyYWJpbGl0eSBpbiBTU0ZzIGluIENoaWxpa2EgTGFnb29uLCBpdCBpcyBpbXBvcnRhbnQgdG8gY29uc2lkZXIgYm90aCBlbnZpcm9ubWVudGFsIGFuZCBzb2NpYWwgZmFjdG9ycywgYXMgd2VsbCBhcyB0aGUgY29tcGxleCBmZWVkYmFjayBsb29wcyBiZXR3ZWVuIHRoZXNlIGZhY3RvcnMuIFRoZSBzdHVkeSBoZWxwcyBpbiBicmlkZ2luZyBhIGNydWNpYWwgZ2FwIGluIG91ciB1bmRlcnN0YW5kaW5nIG9mIHRoZSByb2xlIG9mIHdhdGVyIHF1YWxpdHkgaW4gdnVsbmVyYWJpbGl0eSBhbmFseXNpcyB3aXRoaW4gcmVzb3VyY2UgZGVwZW5kZW50IGNvbW11bml0aWVzLiBXZSBjb25jbHVkZSB3aXRoIGtleSBpbnNpZ2h0cyBvbiBwb3NzaWJsZSBjb3BpbmcgcmVzcG9uc2VzIGFuZCBhZGFwdGl2ZSBjYXBhY2l0eSBuZWNlc3NhcnkgZm9yIHRoZSBzbWFsbC1zY2FsZSBmaXNoZXJpZXMgY29tbXVuaXRpZXMgdG8gdHJhbnNpdGlvbiB0b3dhcmQgdmlhYmlsaXR5LiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzg5L2ZtYXJzLjIwMjMuMTA4NzI5NiIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cuZnJvbnRpZXJzaW4ub3JnL2FydGljbGVzLzEwLjMzODkvZm1hcnMuMjAyMy4xMDg3Mjk2L3BkZiIsImlzc3VlZERhdGUiOiIyMDIzLTA2LTI5IiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiRnJvbnRpZXJzIGluIE1hcmluZSBTY2llbmNlIiwidm9sdW1lIjoxMCwiaW1wYWN0RmFjdG9yIjoiNC4yOCJ9LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJGcm9udGllcnMgTWVkaWEifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjUxLCJjaXRlZEJ5Q291bnQiOjV9fSwicXVvdGUiOiJXYXRlciBxdWFsaXR5IG9mIENoaWxpa2EgTGFnb29uIGhhcyBkZWdyYWRlZCBkdWUgdG8gYW50aHJvcG9nZW5pYyBpbnRlcnZlbnRpb25zIGFuZCBuYXR1cmFsIGZvcmNlcyBvZiBjaGFuZ2UuIFZhcmlvdXMgYWN0aXZpdGllcyBpbiB0aGUgQ2hpbGlrYSBMYWdvb24sIHN1Y2ggYXMgaW5kdXN0cmlhbCB3YXN0ZXdhdGVyIGRpc3Bvc2FsLCBzZXdhZ2UgZHVtcGluZywgYXF1YWN1bHR1cmUsIGh5ZHJvbG9naWNhbCBpbnRlcnZlbnRpb25zLCBhbmQgY3ljbG9uZXMsIGFyZSBwcm9kdWNpbmcgc2FsaW5pdHkgY2hhbmdlcywgc2VkaW1lbnQgZGVwb3NpdGlvbiwgbnV0cmllbnQgZW5yaWNobWVudCwgZXV0cm9waGljYXRpb24sIGFuZCBkZWFkIHpvbmVzLCBhcyBzdGF0ZWQuIEZpc2ggc3Vydml2YWwgaXMgYXQgcmlzayBhcyBhIHJlc3VsdCBvZiB0aGUgc2lnbmlmaWNhbnQgcmVkdWN0aW9uIGluIGVudmlyb25tZW50YWwgY2lyY3Vtc3RhbmNlcywgd2hpY2ggZGltaW5pc2hlcyBmaXNoZXJtZW7igJlzIGluY29tZSBhbmQgcHVzaGVzIHRoZW0gb3V0IG9mIHNvY2lldHkuIFRoZXNlIGNhdXNlcyBhZmZlY3RpbmcgdGhlIHdhdGVyIHF1YWxpdHkgb2YgdGhlIExhZ29vbiBkZXBpY3QgbWFueSBmYWNlcyBvZiB2dWxuZXJhYmlsaXR5IGluIFNTRiBjb21tdW5pdGllcy4gQXMgdGhlIENoaWxpa2EgTGFnb29uIGVudmlyb25tZW50IGlzIGV4cGVjdGVkIHRvIHVuZGVyZ28gaW5jcmVhc2luZ2x5IGRyYXN0aWMgY2hhbmdlcyAoc3VjaCBhcyBjYXRjaCBjb21wb3NpdGlvbiwgY2F0Y2ggY2FwYWNpdHksIGJpb2RpdmVyc2l0eSB2YXJpYXRpb25zLCB3YXRlciBxdWFsaXR5LCBhbmQgZmlzaGVyeSByZXZlbnVlcyksIGludGVybmF0aW9uYWwgcG9saWN5IGhhcyBiZWNvbWUgbW9yZSBpbnRlcmVzdGVkIGluIGFkYXB0YXRpb24gbWVhc3VyZXMuIEdpdmVuIGhvdyBkaXZlcnNlIFNTRiBlY29zeXN0ZW1zIGFyZSBleHBlY3RlZCB0byBiZSBiZXR3ZWVuIGNvbnRpbmVudHMsIHRoZXJlIGlzIG5vIOKAnG9uZSBzaXplIGZpdHMgYWxs4oCdIGFwcHJvYWNoLiBUaGUgcGFwZXIgb3V0bGluZWQgc2V2ZXJhbCBjb3BpbmcgYW5kIGFkYXB0YXRpb24gbWVjaGFuaXNtcyB0byBpbXByb3ZlIHdhdGVyIHF1YWxpdHkgYW5kIGNvbnRpbnVlIGZpc2hpbmcgaW4gYSBzdXN0YWluYWJsZSB3YXkuIEFkYXB0YXRpb24gc3RyYXRlZ2llcyBjYW4gb2NjYXNpb25hbGx5IGhhdmUgdW5mb3Jlc2VlbiBjb25zZXF1ZW5jZXMgZm9yIGZpc2hlcmllcyBhbmQgZmlzaGluZyBjb21tdW5pdGllcy4gQW55IHVuZXhwZWN0ZWQgY29uc2VxdWVuY2VzIG9mIGFkYXB0YXRpb24gY2FuIGJlIG1pdGlnYXRlZCB0aHJvdWdoIGxvbmctdGVybSBwbGFubmluZyBhbmQgaWRlbnRpZmljYXRpb24gb2YgZnV0dXJlIG9wdGlvbnMgdGhyb3VnaCBzY2VuYXJpbyBhbmFseXNpcy4gVGhlIGZpbmRpbmdzIG9mIHRoaXMgcGFwZXIgY2FuIGJlIHV0aWxpemVkIHRvIGRldmVsb3AgcG9saWNpZXMgdG8gcHJvdGVjdCB3YXRlciBxdWFsaXR5IGFuZCBTU0ZzIGluIGEgdmFyaWV0eSBvZiBjb2FzdGFsIGxhZ29vbiBzZXR0aW5ncy4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjIuMjg1MDM2MDg3MDM2MTMzLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDkzMDUzNDM2Mjc5MywiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjMuNzczNTE5NTE1OTkxMjEsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40ODMzNjQxMDUyMjQ2MSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjUuMjU1NTg0NzE2Nzk2ODc1LCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDg5NTU5MTczNTgzOTg0LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyNi43NDQwNjgxNDU3NTE5NTMsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC41MDY4MDU0MTk5MjE4NzUsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjI4LjIyNjEzMzM0NjU1NzYxNywibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ5MDQ2MzI1NjgzNTk0LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyOS43MTQ2MTY3NzU1MTI2OTUsImxlZnQiOjguMzMxNTkyNTU5ODE0NDUzLCJ3aWR0aCI6NDAuNDk0MDk4NjYzMzMwMDgsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjMxLjE5NjY4Mzg4MzY2Njk5MiwibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ5NDM5MjM5NTAxOTUzLCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozMi42ODUxNjU0MDUyNzM0NCwibGVmdCI6OC4zMzE1OTYzNzQ1MTE3MTksIndpZHRoIjo0MC40ODc0NDU4MzEyOTg4MywiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MzQuMTY3MjMyNTEzNDI3NzM0LCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ5Nzc0NTUxMzkxNjAxNiwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MzUuNjU1NzE1OTQyMzgyODEsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDk4NjM4MTUzMDc2MTcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM3LjEzNzc4MzA1MDUzNzExLCJsZWZ0IjoxMC42ODUxODYzODYxMDgzOTgsIndpZHRoIjozOC4xNDExMDU2NTE4NTU0NywiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MzguNjI2MjY2NDc5NDkyMTksImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDg3MTUyMDk5NjA5Mzc1LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo0MC4xMDgzMzM1ODc2NDY0ODQsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDg5ODU2NzE5OTcwNywiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDEuNTk2ODEzMjAxOTA0MywibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40ODg2NjY1MzQ0MjM4MywiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDMuMDc4ODgwMzEwMDU4NTk0LCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4MTg2MTExNDUwMTk1LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo0NC41NjczNjM3MzkwMTM2NywibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40ODMwNTg5Mjk0NDMzNiwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDYuMDQ5NDMwODQ3MTY3OTcsImxlZnQiOjguMzMzMTA4OTAxOTc3NTM5LCJ3aWR0aCI6NDAuNTAwNzQ3NjgwNjY0MDYsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQ3LjUzNzkxNDI3NjEyMzA1LCJsZWZ0Ijo4LjMzMzEwODkwMTk3NzUzOSwid2lkdGgiOjQwLjQ5NTYwNTQ2ODc1LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo0OS4wMTk5ODEzODQyNzczNDQsImxlZnQiOjguMzMzMTA2MDQwOTU0NTksIndpZHRoIjo0MC41MDY4MTY4NjQwMTM2NywiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NTAuNTA4NDY0ODEzMjMyNDIsImxlZnQiOjguMzMzMTA2MDQwOTU0NTksIndpZHRoIjo0MC40ODg1MDYzMTcxMzg2NywiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NTEuOTkwNTI4MTA2Njg5NDUsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40OTc3MjY0NDA0Mjk2OSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NTMuNDc5MDExNTM1NjQ0NTMsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjozMy4xNTk0NDY3MTYzMDg1OTQsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjU0Ljk2MTA3ODY0Mzc5ODgzLCJsZWZ0IjoxMC42ODM2NzAwNDM5NDUzMTIsIndpZHRoIjozOC4xMzY1OTY2Nzk2ODc1LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo1Ni40NDk1NjIwNzI3NTM5MDYsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40ODcxNDQ0NzAyMTQ4NDQsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiMTMifX1d. The major drivers of harmful algal blooms in Indian lakes are summarized in Table 1. Chemically, these macronutrients enter water bodies via surface runoff, groundwater infiltration, and atmospheric deposition, elevating total phosphorus beyond 100 μg/L and total nitrogen above 1 mg/L, far exceeding oligotrophic thresholds (Sitote & Gebremedhine, 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllhYnNyYSBNZWxhayIsImZhbWlseSI6IlNpdG90ZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNaWNoYWVsIEdpcmltYXkiLCJmYW1pbHkiOiJHZWJyZW1lZGhpbmUifX1dLCJpZHMiOnsiaXNzbiI6W10sImlzYm4iOltdLCJkb2kiOiIxMC4yMDk0NC9wcmVwcmludHMyMDI0MDkuMDcwNC52MSIsIm9wZW5hbGV4IjoiVzQ0MDI0NjU0MTQiLCJleGEiOiJodHRwczovL3d3dy5wcmVwcmludHMub3JnL21hbnVzY3JpcHQvMjAyNDA5LjA3MDQifSwidGl0bGUiOiJDb21wcmVoZW5zaXZlIFJldmlldyBvZiBFdXRyb3BoaWNhdGlvbiBpbiBGcmVzaHdhdGVyIEVjb3N5c3RlbXM6IENhdXNlcywgRWZmZWN0cywgQXNzZXNzbWVudCwgYW5kIE1hbmFnZW1lbnQgU3RyYXRlZ2llcyIsImFic3RyYWN0IjoiVGhpcyByZXZpZXcgZXhhbWluZXMgZXV0cm9waGljYXRpb24gaW4gZnJlc2h3YXRlciBlY29zeXN0ZW1zLCBhIHdpZGVzcHJlYWQgZW52aXJvbm1lbnRhbCBpc3N1ZSBzdGVtbWluZyBmcm9tIGV4Y2Vzc2l2ZSBudXRyaWVudCBpbnB1dCwgbGFyZ2VseSBkdWUgdG8gaHVtYW4gYWN0aXZpdGllcy4gVGhlIHN0dWR5IGV4cGxvcmVzIHRoZSBvcmlnaW5zLCBjb25zZXF1ZW5jZXMsIGV2YWx1YXRpb24gdGVjaG5pcXVlcywgYW5kIGNvbnRyb2wgbWV0aG9kcyBmb3IgZXV0cm9waGljYXRpb24uIFByaW1hcnkgY2F1c2VzIGluY2x1ZGUgYWdyaWN1bHR1cmFsIGRpc2NoYXJnZSwgdXJiYW4gZGV2ZWxvcG1lbnQsIGFuZCBjbGltYXRlIHNoaWZ0cy4gVGhlIGVmZmVjdHMgYXJlIGZhci1yZWFjaGluZywgaW1wYWN0aW5nIGJpb2RpdmVyc2l0eSwgd2F0ZXIgcXVhbGl0eSwgYW5kIHNvY2lvLWVjb25vbWljIGZhY3RvcnMuIFRoZSBhc3Nlc3NtZW50IHV0aWxpemVzIGEgY29tYmluYXRpb24gb2YgY2hlbWljYWwsIHBoeXNpY2FsLCBhbmQgYmlvbG9naWNhbCBtYXJrZXJzLCB3aXRoIGN1dHRpbmctZWRnZSB0ZWNobm9sb2dpZXMgbGlrZSByZW1vdGUgc2Vuc2luZyBpbXByb3ZpbmcgbW9uaXRvcmluZyBlZmZvcnRzLiBNaXRpZ2F0aW9uIHN0cmF0ZWdpZXMgYXJlIGRpdmVyc2UsIGluY29ycG9yYXRpbmcgcHJldmVudGF0aXZlIGFjdGlvbnMgKHN1Y2ggYXMgbnV0cmllbnQgbGltaXRhdGlvbiBhbmQgbGFuZCBtYW5hZ2VtZW50KSwgY29ycmVjdGl2ZSBtZWFzdXJlcyAoaW5jbHVkaW5nIGJpby1tYW5pcHVsYXRpb24gYW5kIGNoZW1pY2FsIGFwcGxpY2F0aW9ucyksIGFuZCByZWd1bGF0b3J5IGluaXRpYXRpdmVzIChsaWtlIHdhdGVyIHF1YWxpdHkgZ3VpZGVsaW5lcyBhbmQgbnV0cmllbnQgZXhjaGFuZ2Ugc2NoZW1lcykuIFdoaWxlIGNhc2Ugc3R1ZGllcyBkZW1vbnN0cmF0ZSBzdWNjZXNzZnVsIGludGVydmVudGlvbnMsIHRoZXkgYWxzbyByZXZlYWwgb25nb2luZyBjaGFsbGVuZ2VzLCBwYXJ0aWN1bGFybHkgZWNvc3lzdGVtIHVucHJlZGljdGFiaWxpdHkgYW5kIGNsaW1hdGUgY2hhbmdlIGluZmx1ZW5jZXMuIFRoZSByZXNlYXJjaCBoaWdobGlnaHRzIHRoZSBwcm9taXNlIG9mIGVtZXJnaW5nIHRlY2hub2xvZ2llcywgbm90YWJseSBtYWNoaW5lIGxlYXJuaW5nLCBpbiBlbmhhbmNpbmcgbWFuYWdlbWVudCBwcmFjdGljZXMuIEtub3dsZWRnZSBnYXBzIHBlcnNpc3QsIGVzcGVjaWFsbHkgcmVnYXJkaW5nIGV1dHJvcGhpY2F0aW9uLWJpb2RpdmVyc2l0eSBkeW5hbWljcyBpbiB0cm9waWNhbCBlbnZpcm9ubWVudHMgYW5kIHRoZSBzb2Npby1lY29ub21pYyByYW1pZmljYXRpb25zIG9mIGludGVydmVudGlvbnMuIFRoZSBzdHVkeSBlbXBoYXNpemVzIHRoZSBpbXBvcnRhbmNlIG9mIGZsZXhpYmxlLCBjb21wcmVoZW5zaXZlIGFwcHJvYWNoZXMgdGhhdCBibGVuZCBzY2llbnRpZmljIGluc2lnaHRzIHdpdGggcHJhY3RpY2FsIHNvbHV0aW9ucyBhbmQgc3Ryb25nIHBvbGljeSBzdHJ1Y3R1cmVzIHRvIHRhY2tsZSB0aGlzIGludHJpY2F0ZSBlbnZpcm9ubWVudGFsIGNoYWxsZW5nZSBpbiBhbiBlcmEgb2YgY2xpbWF0ZSBjaGFuZ2UuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjIwOTQ0L3ByZXByaW50czIwMjQwOS4wNzA0LnYxIiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5wcmVwcmludHMub3JnL2Zyb250ZW5kL21hbnVzY3JpcHQvZTA4ZTViNTY1MzNlMmFhZDc0N2ZkOTQzNWU2N2NhNzMvZG93bmxvYWRfcHViIiwiaXNzdWVkRGF0ZSI6IjIwMjQtMDktMTAiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTEyIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJQcmVwcmludHMub3JnIiwiaW1wYWN0RmFjdG9yIjoiMS4xNSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MCwiY2l0ZWRCeUNvdW50Ijo1fX0sInF1b3RlIjoiVGhlIHN0dWR5IGVtcGhhc2l6ZXMgdGhlIGltcG9ydGFuY2Ugb2YgZmxleGlibGUsIGNvbXByZWhlbnNpdmUgYXBwcm9hY2hlcyB0aGF0IGJsZW5kIHNjaWVudGlmaWMgaW5zaWdodHMgd2l0aCBwcmFjdGljYWwgc29sdXRpb25zIGFuZCBzdHJvbmcgcG9saWN5IHN0cnVjdHVyZXMgdG8gdGFja2xlIHRoaXMgaW50cmljYXRlIGVudmlyb25tZW50YWwgY2hhbGxlbmdlIGluIGFuIGVyYSBvZiBjbGltYXRlIGNoYW5nZS4gRXV0cm9waGljYXRpb24sIGNhdXNlZCBieSBleGNlc3NpdmUgbnV0cmllbnQgZW5yaWNobWVudCBpbiB3YXRlciBib2RpZXMsIGlzIGEgc2lnbmlmaWNhbnQgZW52aXJvbm1lbnRhbCBjaGFsbGVuZ2UgZm9yIGZyZXNod2F0ZXIgZWNvc3lzdGVtcyBvbiBhIGdsb2JhbCBzY2FsZS4gUHJpbWFyaWx5IGRyaXZlbiBieSBuaXRyb2dlbiBhbmQgcGhvc3Bob3J1cyBpbnB1dHMsIHRoaXMgcHJvY2VzcyByZXN1bHRzIGluIHRoZSBvdmVycHJvZHVjdGlvbiBvZiBhbGdhZSBhbmQgYXF1YXRpYyBwbGFudHMsIGxlYWRpbmcgdG8gYSBjYXNjYWRlIG9mIGVjb2xvZ2ljYWwgZGlzdHVyYmFuY2VzLCBpbmNsdWRpbmcgb3h5Z2VuIGRlcGxldGlvbiAoaHlwb3hpYSksIGxvc3Mgb2YgYmlvZGl2ZXJzaXR5LCBhbmQgc2V2ZXJlIHdhdGVyIHF1YWxpdHkgZGVncmFkYXRpb24gKEtha2FkZSBldCBhbC4sIDIwMjE7IE1lZXJob2ZmIGV0IGFsLiwgMjAyMjsgV3VydHNiYXVnaCBldCBhbC4sIDIwMTkpLiJ9XQ%3D%3D. This nutrient overload disrupts the natural phosphorus-nitrogen balance, favoring phytoplankton proliferation. In stagnant, warm waters (25-35°C) typical of Indian reservoirs, phosphorus acts as the limiting factor in many systems, binding to sediments until resuspended by wind or bioturbation, triggering rapid algal growth (Jilbert et al., 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRvbSIsImZhbWlseSI6IkppbGJlcnQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUmFvdWzigJBNYXJpZSIsImZhbWlseSI6IkNvdXR1cmUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQnJpYW4iLCJmYW1pbHkiOiJIdXNlciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLYWxldmkiLCJmYW1pbHkiOiJTYWxvbmVuIn19XSwiaWRzIjp7Imlzc24iOlsiMDAxOC04MTU4IiwiMTU3My01MTE3Il0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDA3L3MxMDc1MC0wMjAtMDQ0NTcteCIsIm1hZyI6IjMxMDA0NTkwNTAiLCJvcGVuYWxleCI6IlczMTAwNDU5MDUwIn0sInRpdGxlIjoiUHJlZmFjZTogUmVzdG9yYXRpb24gb2YgZXV0cm9waGljIGxha2VzOiBjdXJyZW50IHByYWN0aWNlcyBhbmQgZnV0dXJlIGNoYWxsZW5nZXMiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAwNy9zMTA3NTAtMDIwLTA0NDU3LXgiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vbGluay5zcHJpbmdlci5jb20vY29udGVudC9wZGYvMTAuMTAwNy9zMTA3NTAtMDIwLTA0NDU3LXgucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjAtMTEtMTYiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA3IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJIeWRyb2Jpb2xvZ2lhIiwiaXNzdWUiOjIxLCJ2b2x1bWUiOjg0NywiaW1wYWN0RmFjdG9yIjoiMy4wMCJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjQzNDMsImxhc3RQYWdlIjo0MzU3fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlNwcmluZ2VyIFNjaWVuY2UrQnVzaW5lc3MgTWVkaWEifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjk1LCJjaXRlZEJ5Q291bnQiOjU4fX0sInF1b3RlIjoiU2ltaWxhciBkZWdyYWRhdGlvbiBvZiBmcmVzaHdhdGVyIGVjb3N5c3RlbXMgaXMgb2JzZXJ2ZWQgd29ybGR3aWRlLCB3aXRoIHRoZSB0aW1lbGluZSBvZiBodW1hbiBwcmVzc3VyZSBkaWN0YXRlZCBieSB0cmVuZHMgaW4gdXJiYW5pemF0aW9uIGFuZCBpbnRlbnNpdmUgYWdyaWN1bHR1cmUgaW4gZGlmZmVyZW50IGdlb2dyYXBoaWMgcmVnaW9ucyAoZS5nLiwgVmVubmVtbyBldCBhbC4sIDIwMDkpLiBUaGUgcHJpbmNpcGFsIGRyaXZlciBvZiBlY29zeXN0ZW0gc3RyZXNzIGluIG1vc3QgYXF1YXRpYyBzeXN0ZW1zIGlzIGV1dHJvcGhpY2F0aW9uLCB0aGUgZXhjZXNzIGdyb3d0aCBvZiBhbGdhZSBkcml2ZW4gYnkgYW50aHJvcG9nZW5pYyBpbnB1dHMgb2YgdGhlIG1hY3JvbnV0cmllbnRzIG5pdHJvZ2VuIChOKSBhbmQgcGhvc3Bob3J1cyAoUCkgKFNtaXRoLCAyMDAzOyBCaXJrIGV0IGFsLiwgMjAyMCkuIE51dHJpZW50cyBhcmUgZW1pdHRlZCBmcm9tIGJvdGggcG9pbnQgYW5kIGRpZmZ1c2Ugc291cmNlcyBpbiB0aGUgaHVtYW4gZW52aXJvbm1lbnQgYW5kIGVudGVyIGFxdWF0aWMgc3lzdGVtcyB2aWEgc3VyZmFjZSBydW5vZmYsIGFzIHdlbGwgYXMgZ3JvdW5kLXdhdGVyIGFuZCBhdG1vc3BoZXJpYyBkZXBvc2l0aW9uIChQYWVybCwgMTk5NykuIEV1dHJvcGhpY2F0aW9uIG9mdGVuIGxlYWRzIHRvIHRoZSBkb21pbmFuY2Ugb2YgcHJpbWFyeSBwcm9kdWN0aW9uIGJ5IGN5YW5vYmFjdGVyaWEsIGluY2x1ZGluZyB0b3hpYyBnZW5lcmEgc3VjaCBhcyBEb2xpY2hvc3Blcm11bSwgQXBoYW5pem9tZW5vbiwgYW5kIE1pY3JvY3lzdGlzIChIdWlzbWFuIGV0IGFsLiwgMjAxOCksIGFzIHdlbGwgYXMgb3h5Z2VuIGRlcGxldGlvbiBhbmQgaGFiaXRhdCBkZWdyYWRhdGlvbiBpbiBsYWtlcyAoQXJlbmQgZXQgYWwuLCAyMDExKS4gQmVjYXVzZSBsYWtlcyBwcm92aWRlIGEgcmFuZ2Ugb2YgY3J1Y2lhbCBlY29zeXN0ZW0gc2VydmljZXMgKFdpbHNvbiAmIENhcnBlbnRlciwgMTk5OSksIHRoZXJlIGlzIGEgZ3Jvd2luZyBuZWVkIGZvciBtYW5hZ2VtZW50IGFwcHJvYWNoZXMgdGhhdCByZXN0b3JlIHdhdGVyIHF1YWxpdHkgaW4gZnJlc2h3YXRlciBzeXN0ZW1zLiBXaXRoaW4gdGhlIGZyYW1ld29yayBvZiBVbml0ZWQgTmF0aW9ucyBTdXN0YWluYWJsZSBEZXZlbG9wLW1lbnQgR29hbCA2IChDbGVhbiBXYXRlciBhbmQgU2FuaXRhdGlvbiksIHNldmVyYWwgZGlzdGluY3QgdGFyZ2V0cyByZWxhdGUgdG8gdGhlIGltcHJvdmVtZW50IG9mIHdhdGVyIHF1YWxpdHkgYW5kIHByb3RlY3Rpb24gb2YgYXF1YXRpYyBlY29zeXN0ZW1zIGJ5IDIwMzAgKFVuaXRlZCBOYXRpb25zLCAyMDE1KS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyOC4yOTg4NjQzNjQ2MjQwMjMsImxlZnQiOjguNTQ5MjA1NzgwMDI5Mjk3LCJ3aWR0aCI6MzkuMzcxNDk0MjkzMjEyODksImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MjkuOTkxMTM0NjQzNTU0Njg4LCJsZWZ0Ijo4LjU0OTIwNTc4MDAyOTI5Nywid2lkdGgiOjM5LjM2NTExOTkzNDA4MjAzLCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMxLjY4MzQwNjgyOTgzMzk4NCwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNjk2NjMyMzg1MjUzOSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozMy4zNjc5NzMzMjc2MzY3MiwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNzU2NzkwMTYxMTMyOCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNS4wNjAyNDkzMjg2MTMyOCwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNzA5NjQwNTAyOTI5NywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNi43NTI1MTc3MDAxOTUzMSwibGVmdCI6OC41NDkyMDg2NDEwNTIyNDYsIndpZHRoIjozOS4zNzY5NjA3NTQzOTQ1MywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC40MzcwODgwMTI2OTUzMSwibGVmdCI6OC41NDkyMDg2NDEwNTIyNDYsIndpZHRoIjozOS4zNjY1Njk1MTkwNDI5NywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0MC4xMjkzNTYzODQyNzczNDQsImxlZnQiOjguNTQ5MjA4NjQxMDUyMjQ2LCJ3aWR0aCI6MzkuMzY0Mzc5ODgyODEyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0MS44MjE2MjQ3NTU4NTkzNzUsImxlZnQiOjguNTQ5MjA4NjQxMDUyMjQ2LCJ3aWR0aCI6MzkuMzY1NjczMDY1MTg1NTUsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDMuNTA2MTk1MDY4MzU5Mzc1LCJsZWZ0Ijo4LjU0OTM4OTgzOTE3MjM2Mywid2lkdGgiOjM5LjM3MDM5MTg0NTcwMzEyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0NS4xOTg0NjM0Mzk5NDE0MDYsImxlZnQiOjguNTQ5NTcyOTQ0NjQxMTEzLCJ3aWR0aCI6MzkuMzY1ODQ0NzI2NTYyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ni44OTA3MzU2MjYyMjA3LCJsZWZ0Ijo4LjU0OTU3Mjk0NDY0MTExMywid2lkdGgiOjM5LjM3Mjk1NTMyMjI2NTYyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0OC41NzUzMDIxMjQwMjM0NCwibGVmdCI6OC41NDk1NzI5NDQ2NDExMTMsIndpZHRoIjozOS4zNzI0MjEyNjQ2NDg0NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1MC4yNjc1NzgxMjUsImxlZnQiOjguNTQ5NTc2NzU5MzM4Mzc5LCJ3aWR0aCI6MzkuMzcxMTI0MjY3NTc4MTI1LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjUxLjk1OTg0MjY4MTg4NDc2NiwibGVmdCI6OC41NDk1NzY3NTkzMzgzNzksIndpZHRoIjozOS4zNjc4MzU5OTg1MzUxNTYsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTMuNTc0MTE5NTY3ODcxMDk0LCJsZWZ0Ijo4LjU0OTU3Njc1OTMzODM3OSwid2lkdGgiOjM5LjM3MDc3NzEzMDEyNjk1LCJoZWlnaHQiOjEuMzM2ODk1NTg1MDYwMTE5Nn19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjU1LjI2NjM5MTc1NDE1MDM5LCJsZWZ0Ijo4LjU0OTc2MDgxODQ4MTQ0NSwid2lkdGgiOjM5LjM2NjU4MDk2MzEzNDc2NiwiaGVpZ2h0IjoxLjMzNjg5NTU4NTA2MDExOTZ9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1Ny4wMjg5NTM1NTIyNDYwOTQsImxlZnQiOjguNTQ5OTQ0ODc3NjI0NTEyLCJ3aWR0aCI6MzkuMzYyMDI2MjE0NTk5NjEsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTguNzEzNTIzODY0NzQ2MDk0LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjkuMjk5OTgyMDcwOTIyODUyLCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjYwLjQwNTc5MjIzNjMyODEyNSwibGVmdCI6MTAuNjIyNDU1NTk2OTIzODI4LCJ3aWR0aCI6MzcuMjg3ODY4NDk5NzU1ODYsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NjIuMDk4MDYwNjA3OTEwMTU2LCJsZWZ0Ijo4LjU0OTk0MzkyMzk1MDE5NSwid2lkdGgiOjM5LjM3MzMyNTM0NzkwMDM5LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjYzLjc4MjYzMDkyMDQxMDE1NiwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzMxMjY5ODM2NDI1OCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2NS40NzQ5MDY5MjEzODY3MiwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNjUxMTk5MzQwODIwMywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2Ny4xNjcxNzUyOTI5Njg3NSwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNjk0ODAxMzMwNTY2NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2OC43ODE0NDgzNjQyNTc4MSwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzczMzg0MDk0MjM4MywiaGVpZ2h0IjoxLjMzNjg5MTUzMTk0NDI3NX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjcwLjU0NDAxMzk3NzA1MDc4LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjM5LjM2OTMwODQ3MTY3OTY5LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjcyLjIzNjI4OTk3ODAyNzM0LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjM5LjM3MjAzMjE2NTUyNzM0NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo3My45MjA4NDUwMzE3MzgyOCwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzEzMDczNzMwNDY4NzUsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNDM0NCJ9fV0%3D. The process initiates when soluble reactive phosphorus and ammonium ions become bioavailable, assimilated by algae through active uptake mechanisms, converting inorganic nutrients into organic biomass via photosynthesis.

Table 1: Causes of Harmful Algal Blooms in Indian Lakes

| Cause | Description |
| --- | --- |
| Eutrophication | Excessive nutrient enrichment, primarily nitrogen and phosphorus, from anthropogenic sources |
| Agricultural Runoff | Fertilizers, pesticides, and monsoon washout leading to nutrient loading |
| Urban Sewage and Industrial Discharge | Untreated wastewater containing high levels of organic matter and nutrients |
| Climate Change | Rising temperatures and altered monsoon patterns favoring cyanobacterial growth |

Biologically, nutrient enrichment stimulates explosive phytoplankton multiplication, shifting lake dynamics from oligotrophic clarity to hypertrophic turbidity. Excessive N and P fuel primary production, with algae doubling biomass in days under optimal light and temperature, forming dense blooms visible as green scums (Sitote & Gebremedhine, 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllhYnNyYSBNZWxhayIsImZhbWlseSI6IlNpdG90ZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNaWNoYWVsIEdpcmltYXkiLCJmYW1pbHkiOiJHZWJyZW1lZGhpbmUifX1dLCJpZHMiOnsiaXNzbiI6W10sImlzYm4iOltdLCJkb2kiOiIxMC4yMDk0NC9wcmVwcmludHMyMDI0MDkuMDcwNC52MSIsIm9wZW5hbGV4IjoiVzQ0MDI0NjU0MTQiLCJleGEiOiJodHRwczovL3d3dy5wcmVwcmludHMub3JnL21hbnVzY3JpcHQvMjAyNDA5LjA3MDQifSwidGl0bGUiOiJDb21wcmVoZW5zaXZlIFJldmlldyBvZiBFdXRyb3BoaWNhdGlvbiBpbiBGcmVzaHdhdGVyIEVjb3N5c3RlbXM6IENhdXNlcywgRWZmZWN0cywgQXNzZXNzbWVudCwgYW5kIE1hbmFnZW1lbnQgU3RyYXRlZ2llcyIsImFic3RyYWN0IjoiVGhpcyByZXZpZXcgZXhhbWluZXMgZXV0cm9waGljYXRpb24gaW4gZnJlc2h3YXRlciBlY29zeXN0ZW1zLCBhIHdpZGVzcHJlYWQgZW52aXJvbm1lbnRhbCBpc3N1ZSBzdGVtbWluZyBmcm9tIGV4Y2Vzc2l2ZSBudXRyaWVudCBpbnB1dCwgbGFyZ2VseSBkdWUgdG8gaHVtYW4gYWN0aXZpdGllcy4gVGhlIHN0dWR5IGV4cGxvcmVzIHRoZSBvcmlnaW5zLCBjb25zZXF1ZW5jZXMsIGV2YWx1YXRpb24gdGVjaG5pcXVlcywgYW5kIGNvbnRyb2wgbWV0aG9kcyBmb3IgZXV0cm9waGljYXRpb24uIFByaW1hcnkgY2F1c2VzIGluY2x1ZGUgYWdyaWN1bHR1cmFsIGRpc2NoYXJnZSwgdXJiYW4gZGV2ZWxvcG1lbnQsIGFuZCBjbGltYXRlIHNoaWZ0cy4gVGhlIGVmZmVjdHMgYXJlIGZhci1yZWFjaGluZywgaW1wYWN0aW5nIGJpb2RpdmVyc2l0eSwgd2F0ZXIgcXVhbGl0eSwgYW5kIHNvY2lvLWVjb25vbWljIGZhY3RvcnMuIFRoZSBhc3Nlc3NtZW50IHV0aWxpemVzIGEgY29tYmluYXRpb24gb2YgY2hlbWljYWwsIHBoeXNpY2FsLCBhbmQgYmlvbG9naWNhbCBtYXJrZXJzLCB3aXRoIGN1dHRpbmctZWRnZSB0ZWNobm9sb2dpZXMgbGlrZSByZW1vdGUgc2Vuc2luZyBpbXByb3ZpbmcgbW9uaXRvcmluZyBlZmZvcnRzLiBNaXRpZ2F0aW9uIHN0cmF0ZWdpZXMgYXJlIGRpdmVyc2UsIGluY29ycG9yYXRpbmcgcHJldmVudGF0aXZlIGFjdGlvbnMgKHN1Y2ggYXMgbnV0cmllbnQgbGltaXRhdGlvbiBhbmQgbGFuZCBtYW5hZ2VtZW50KSwgY29ycmVjdGl2ZSBtZWFzdXJlcyAoaW5jbHVkaW5nIGJpby1tYW5pcHVsYXRpb24gYW5kIGNoZW1pY2FsIGFwcGxpY2F0aW9ucyksIGFuZCByZWd1bGF0b3J5IGluaXRpYXRpdmVzIChsaWtlIHdhdGVyIHF1YWxpdHkgZ3VpZGVsaW5lcyBhbmQgbnV0cmllbnQgZXhjaGFuZ2Ugc2NoZW1lcykuIFdoaWxlIGNhc2Ugc3R1ZGllcyBkZW1vbnN0cmF0ZSBzdWNjZXNzZnVsIGludGVydmVudGlvbnMsIHRoZXkgYWxzbyByZXZlYWwgb25nb2luZyBjaGFsbGVuZ2VzLCBwYXJ0aWN1bGFybHkgZWNvc3lzdGVtIHVucHJlZGljdGFiaWxpdHkgYW5kIGNsaW1hdGUgY2hhbmdlIGluZmx1ZW5jZXMuIFRoZSByZXNlYXJjaCBoaWdobGlnaHRzIHRoZSBwcm9taXNlIG9mIGVtZXJnaW5nIHRlY2hub2xvZ2llcywgbm90YWJseSBtYWNoaW5lIGxlYXJuaW5nLCBpbiBlbmhhbmNpbmcgbWFuYWdlbWVudCBwcmFjdGljZXMuIEtub3dsZWRnZSBnYXBzIHBlcnNpc3QsIGVzcGVjaWFsbHkgcmVnYXJkaW5nIGV1dHJvcGhpY2F0aW9uLWJpb2RpdmVyc2l0eSBkeW5hbWljcyBpbiB0cm9waWNhbCBlbnZpcm9ubWVudHMgYW5kIHRoZSBzb2Npby1lY29ub21pYyByYW1pZmljYXRpb25zIG9mIGludGVydmVudGlvbnMuIFRoZSBzdHVkeSBlbXBoYXNpemVzIHRoZSBpbXBvcnRhbmNlIG9mIGZsZXhpYmxlLCBjb21wcmVoZW5zaXZlIGFwcHJvYWNoZXMgdGhhdCBibGVuZCBzY2llbnRpZmljIGluc2lnaHRzIHdpdGggcHJhY3RpY2FsIHNvbHV0aW9ucyBhbmQgc3Ryb25nIHBvbGljeSBzdHJ1Y3R1cmVzIHRvIHRhY2tsZSB0aGlzIGludHJpY2F0ZSBlbnZpcm9ubWVudGFsIGNoYWxsZW5nZSBpbiBhbiBlcmEgb2YgY2xpbWF0ZSBjaGFuZ2UuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjIwOTQ0L3ByZXByaW50czIwMjQwOS4wNzA0LnYxIiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5wcmVwcmludHMub3JnL2Zyb250ZW5kL21hbnVzY3JpcHQvZTA4ZTViNTY1MzNlMmFhZDc0N2ZkOTQzNWU2N2NhNzMvZG93bmxvYWRfcHViIiwiaXNzdWVkRGF0ZSI6IjIwMjQtMDktMTAiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTEyIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJQcmVwcmludHMub3JnIiwiaW1wYWN0RmFjdG9yIjoiMS4xNSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MCwiY2l0ZWRCeUNvdW50Ijo1fX0sInF1b3RlIjoiVGhlIHN0dWR5IGVtcGhhc2l6ZXMgdGhlIGltcG9ydGFuY2Ugb2YgZmxleGlibGUsIGNvbXByZWhlbnNpdmUgYXBwcm9hY2hlcyB0aGF0IGJsZW5kIHNjaWVudGlmaWMgaW5zaWdodHMgd2l0aCBwcmFjdGljYWwgc29sdXRpb25zIGFuZCBzdHJvbmcgcG9saWN5IHN0cnVjdHVyZXMgdG8gdGFja2xlIHRoaXMgaW50cmljYXRlIGVudmlyb25tZW50YWwgY2hhbGxlbmdlIGluIGFuIGVyYSBvZiBjbGltYXRlIGNoYW5nZS4gRXV0cm9waGljYXRpb24sIGNhdXNlZCBieSBleGNlc3NpdmUgbnV0cmllbnQgZW5yaWNobWVudCBpbiB3YXRlciBib2RpZXMsIGlzIGEgc2lnbmlmaWNhbnQgZW52aXJvbm1lbnRhbCBjaGFsbGVuZ2UgZm9yIGZyZXNod2F0ZXIgZWNvc3lzdGVtcyBvbiBhIGdsb2JhbCBzY2FsZS4gUHJpbWFyaWx5IGRyaXZlbiBieSBuaXRyb2dlbiBhbmQgcGhvc3Bob3J1cyBpbnB1dHMsIHRoaXMgcHJvY2VzcyByZXN1bHRzIGluIHRoZSBvdmVycHJvZHVjdGlvbiBvZiBhbGdhZSBhbmQgYXF1YXRpYyBwbGFudHMsIGxlYWRpbmcgdG8gYSBjYXNjYWRlIG9mIGVjb2xvZ2ljYWwgZGlzdHVyYmFuY2VzLCBpbmNsdWRpbmcgb3h5Z2VuIGRlcGxldGlvbiAoaHlwb3hpYSksIGxvc3Mgb2YgYmlvZGl2ZXJzaXR5LCBhbmQgc2V2ZXJlIHdhdGVyIHF1YWxpdHkgZGVncmFkYXRpb24gKEtha2FkZSBldCBhbC4sIDIwMjE7IE1lZXJob2ZmIGV0IGFsLiwgMjAyMjsgV3VydHNiYXVnaCBldCBhbC4sIDIwMTkpLiJ9XQ%3D%3D. Cyanobacteria like Microcystis aeruginosa, Dolichospermum, and Aphanizomenon dominate due to physiological adaptations: gas vacuoles enable buoyancy regulation for surface scum formation, blocking light to submerged macrophytes; nitrogen-fixing capabilities via heterocysts allow N-limited dominance; and allelopathic toxin release inhibits competitors (Çelekli̇, 2024; Jilbert et al., 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFidXplciIsImZhbWlseSI6IsOHZWxla2xpzIcifX1dLCJpZHMiOnsiaXNzbiI6W10sImlzYm4iOltdLCJkb2kiOiIxMC4yMTIwMy9ycy4zLnJzLTUwNzgxNzYvdjEiLCJvcGVuYWxleCI6Ilc0NDA0NTI5NDE4In0sInRpdGxlIjoiVGhlIGZpcnN0IGVjby1hc3Nlc3NtZW50IG9mIHBoeXRvcGxhbmt0b24gYXNzZW1ibGFnZXMgYXNzb2NpYXRlZCB3aXRoIGVudmlyb25tZW50YWwgY29uZGl0aW9ucyBvZiBkaWZmZXJlbnQgbGFrZXMgaW4gdGhlIFdlc3Rlcm4gQmxhY2sgU2VhIGJhc2luIG9mIFTDvHJraXllIiwiYWJzdHJhY3QiOiI8dGl0bGU%2BQWJzdHJhY3Q8L3RpdGxlPiBMYWtlcyBwbGF5IGEgdml0YWwgcm9sZSBpbiBzdXBwb3J0aW5nIGJpb2RpdmVyc2l0eSwgcHJvdmlkaW5nIHdhdGVyIHJlc291cmNlcywgcmVndWxhdGluZyBjbGltYXRlLCBjeWNsaW5nIG51dHJpZW50cywgYW5kIG9mZmVyaW5nIHJlY3JlYXRpb25hbCBvcHBvcnR1bml0aWVzLiBEZXNwaXRlIHRoZWlyIGltcG9ydGFuY2UgZm9yIGVudmlyb25tZW50YWwgaGVhbHRoIGFuZCBodW1hbiB3ZWxsLWJlaW5nLCBsYWtlcyBmYWNlIHNpZ25pZmljYW50IHByZXNzdXJlcyBpbiB0aGUgQW50aHJvcG9jZW5lIGVyYS4gVGhlIHByZXNlbnQgd29yayBzZWVrcyB0byBhc3Nlc3MgdGhlIHNwZWNpZXMtZW52aXJvbm1lbnQgaW50ZXJhY3Rpb25zIGFuZCB0aGUgZWNvbG9naWNhbCBzdGF0dXMgb2Ygc2l4IGxha2VzIGluIHRoZSBXZXN0ZXJuIEJsYWNrIFNlYSBiYXNpbiBvZiBUw7xya2l5ZSB1dGlsaXppbmcgcGh5dG9wbGFua3RvbiBtZXRyaWNzIGR1cmluZyB3ZXQgYW5kIGRyeSBwZXJpb2RzLiBDYW5vbmljYWwgY29ycmVzcG9uZGVuY2UgYW5hbHlzaXMgcmV2ZWFsZWQgYSBzaWduaWZpY2FudCBjb3JyZWxhdGlvbiBlcXVhbCB0byA5OC41JSBiZXR3ZWVuIHBoeXRvcGxhbmt0b24gc3BlY2llcyBhbmQgZW52aXJvbm1lbnRhbCBzdHJlc3NvcnMgZHVyaW5nIHR3byBoeWRyb2xvZ2ljYWwgcGVyaW9kcy4gRWxlY3RyaWNhbCBjb25kdWN0aXZpdHkgKEVDKSwgcEgsIHRvdGFsIG9yZ2FuaWMgY2FyYm9uIChUT0MpLCBhbmQgdGVtcGVyYXR1cmUgd2VyZSB0aGUgbW9zdCBpbmZsdWVudGlhbCBlbnZpcm9ubWVudGFsIGZhY3RvcnMgYWZmZWN0aW5nIHBoeXRvcGxhbmt0b24gZGlzdHJpYnV0aW9uIGluIGxha2VzIChwPTAuMDAyKS4gTGFrZSBTYXLEsWt1bSwgYSBicmFja2lzaCBoYWJpdGF0LCB3YXMgdW5kZXIgaGlnaCBFQyBhbmQgcEggcHJlc3N1cmUgYW5kIGlzIGNoYXJhY3Rlcml6ZWQgYnkgcG9sbHV0aW9uLXRvbGVyYW50IHNwZWNpZXMuIExha2UgWWVuacOnYcSfYSBhc3NvY2lhdGVkIHdpdGggVE9DIGlzIGNoYXJhY3Rlcml6ZWQgYnkgc29tZSBzcGVjaWVzLCBzdWNoIGFzIDxpdGFsaWM%2BQW5hYmFlbm9wc2lzPC9pdGFsaWM%2BIDxpdGFsaWM%2BbWlsbGVyaTwvaXRhbGljPiwgPGl0YWxpYz5DaHJvb2NvY2N1czwvaXRhbGljPiA8aXRhbGljPnR1cmdpZHVzPC9pdGFsaWM%2BLDxpdGFsaWM%2BIFBzZXVkb3NjaHJvZWRlcmlhPC9pdGFsaWM%2BIDxpdGFsaWM%2Bcm9idXN0YTwvaXRhbGljPiwgPGl0YWxpYz5BcGhhbm9jYXBzYTwvaXRhbGljPiBzcC4sIDxpdGFsaWM%2BTWVyaXNtb3BlZGlhPC9pdGFsaWM%2BIDxpdGFsaWM%2BZ2xhdWNhPC9pdGFsaWM%2BLCA8aXRhbGljPk1pY3JhY3Rpbml1bTwvaXRhbGljPiA8aXRhbGljPnF1YWRyaXNldHVtPC9pdGFsaWM%2BLCBhbmQ8aXRhbGljPiBNaWNyb2N5c3RpczwvaXRhbGljPiA8aXRhbGljPmFlcnVnaW5vc2E8L2l0YWxpYz4uIExha2UgQWJhbnQgaXMgbG9jYXRlZCBvbiB0aGUgb3Bwb3NpdGUgc2lkZSBvZiBFQywgVE9DLCBhbmQgdGVtcGVyYXR1cmUsIHdoaWNoIHdhcyBhc3NvY2lhdGVkIHdpdGggc29tZSBzcGVjaWVzIHN1Y2ggYXMgPGl0YWxpYz5DeW1iZWxsYSBhZmZpbmlzPC9pdGFsaWM%2BLCA8aXRhbGljPkFjaG5hbnRoaWRpdW08L2l0YWxpYz4gPGl0YWxpYz5taW51dGlzc2ltdW08L2l0YWxpYz4sIDxpdGFsaWM%2BRW5jeW9uZW1hPC9pdGFsaWM%2BIDxpdGFsaWM%2BbWludXR1bTwvaXRhbGljPiwgPGl0YWxpYz5FLjwvaXRhbGljPiA8aXRhbGljPnNpbGVzaWFjdW08L2l0YWxpYz4sIGFuZDxpdGFsaWM%2BIERpbm9icnlvbjwvaXRhbGljPiA8aXRhbGljPmRpdmVyZ2VuczwvaXRhbGljPi4gUmVzdWx0cyBvZiB0aGUgcGh5dG8tYXNzZXNzbWVudCBkaXNwbGF5ZWQgdGhhdCB0aGUgZWNvbG9naWNhbCBzdGF0dXMgKEVTKSBvZiB0aGUgc2FtcGxpbmcgc3RhdGlvbnMgZHVyaW5nIHRoZSByYWlueSBhbmQgZHJ5IHBlcmlvZHMgdmFyaWVkIGZyb20gYmFkIHRvIGhpZ2guIFRoZSBtb2RpZmllZCBwaHl0b3BsYW5rdG9uIHRyb3BoaWMgaW5kZXggKE1QVEkpIGV4aGliaXRlZCB0aGF0IGEgbW9kZXJhdGUgRVMgd2FzIGZvdW5kIGluIExha2VzIFNhcsSxa3VtIGFuZCBZZW5pw6dhxJ9hLCB3aGlsZSBvdGhlcnMgaGFkIGEgZ29vZCBFUy4gVGhlIHByZXNlbnQgc3R1ZHkgY29u76yBcm1lZCB0aGF0IHBoeXRvcGxhbmt0b24gY29tbXVuaXRpZXMgYXJlIHN0cm9uZ2x5IGxpbmtlZCB0byB0aGUgZWNvbG9naWNhbCBzdGF0dXMgb2YgbGFrZXMgaW4gdGhlIFdlc3Rlcm4gQmxhY2sgU2VhIGJhc2luLCB3aGljaCBjb3VsZCBiZSBhc3Nlc3NlZCB1c2luZyB0aGUgTVBUSS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjEyMDMvcnMuMy5ycy01MDc4MTc2L3YxIiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5yZXNlYXJjaHNxdWFyZS5jb20vYXJ0aWNsZS9ycy01MDc4MTc2L2xhdGVzdC5wZGYiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMS0xOSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDgiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IlJlc2VhcmNoIFNxdWFyZSAoUmVzZWFyY2ggU3F1YXJlKSJ9LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJSZXNlYXJjaCBTcXVhcmUgKFVuaXRlZCBTdGF0ZXMpIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo1MiwiY2l0ZWRCeUNvdW50IjowfX0sInF1b3RlIjoiRGVzcGl0ZSB0aGVpciBlY29sb2dpY2FsIHNpZ25p77%2B9Y2FuY2UsIHRoZXNlIHdhdGVyIGJvZGllcyBhcmUgaW5jcmVhc2luZ2x5IHRocmVhdGVuZWQgYnkgYW50aHJvcG9nZW5pYyBhY3Rpdml0aWVzIChlLmcuLCB1cmJhbml6YXRpb24sIGFncmljdWx0dXJhbCBsYW5kIHVzZSwgd2FzdGV3YXRlciBkaXNjaGFyZ2UsIHJlZWQgY3V0dGluZywgc2FuZCByZW1vdmFsLCBldGMuKSBhbmQgY2xpbWF0ZSBjaGFuZ2UgKEjDpGRlciBldCBhbC4gMjAyMDsgw4dlbGVrbGkgZXQgYWwuIDIwMjQpLiBOdXRyaWVudCBlbnJpY2htZW50IGluIGxlbnRpYyBlY29zeXN0ZW1zIGltcGFjdHMgbm90IG9ubHkgdGhlIHBoeXNpY29jaGVtaWNhbCBmYWN0b3JzIG9mIGxha2VzIGJ1dCBhbHNvIG5lZ2F0aXZlbHkgYWZmZWN0cyBhcXVhdGljIGxpZmUsIGNhdXNpbmcgaGFybWZ1bCBhbGdhbCBibG9vbXMsIGV4Y2Vzc2l2ZSBncm93dGggb2YgbnVpc2FuY2UgbWFjcm9waHl0ZXMsIGFuZCBpbmNyZWFzZWQg77%2B9c2ggbW9ydGFsaXR5LCBlc3BlY2lhbGx5IHNlbnNpdGl2ZSBzcGVjaWVzIChTdXJlc2ggZXQgYWwuIDIwMjMpLiBJdCBhbHNvIGRpc3J1cHRzIHRoZSBoeWRyb21vcnBob2xvZ3kgYW5kIG92ZXJhbGwgZnVuY3Rpb25pbmcgb2YgdGhlc2UgZWNvc3lzdGVtcy4gRGVzcGl0ZSBnbG9iYWwgd2F0ZXIgc2hvcnRhZ2VzLCB0aGUgZ3Jvd2luZyBkZWdyYWRhdGlvbiBvZiBlY29zeXN0ZW1zIGFuZCB0aGUgb3ZlcnVzZSBvZiBmcmVzaHdhdGVyIHJlc291cmNlcyBoYXZlIHNoaWZ0ZWQgYXR0ZW50aW9uIGJhY2sgdG8gZnJlc2h3YXRlciBlY29zeXN0ZW1zIHRvIGJldHRlciBtYW5hZ2UgYW5kIHByZXNlcnZlIHdhdGVyIHJlc291cmNlcyBnbG9iYWxseSAow4dlbGVrbGkgZXQgYWwuIDIwMjE7IFN1cmVzaCBldCBhbC4gMjAyMykuIEFzc2Vzc2luZyB0aGUgZWNvbG9naWNhbCBzdGF0dXMgb2YgbGFrZXMgdGhyb3VnaCBwaHl0b3BsYW5rdG9uIG1ldHJpY3MgaXMgYSBjcnVjaWFsIGNvbXBvbmVudCBvZiBlbnZpcm9ubWVudGFsIG1vbml0b3JpbmcgYW5kIG1hbmFnZW1lbnQsIGFzIGl0IGFsaWducyB3aXRoIHRoZSBndWlkZWxpbmVzIHNldCBieSB0aGUgV2F0ZXIgRnJhbWV3b3JrIERpcmVjdGl2ZSAoRGlyZWN0aXZlIDIwMDApLiBQaHl0b3BsYW5rdG9uLCBhcyBwcmltYXJ5IHByb2R1Y2VycywgYXJlIHNlbnNpdGl2ZSBpbmRpY2F0b3JzIG9mIGNoYW5nZXMgaW4gd2F0ZXIgcXVhbGl0eSBhbmQgZWNvc3lzdGVtIGhlYWx0aCAoUmV5bm9sZHMgZXQgYWwuIDIwMDI7IMOHZWxla2xpIGV0IGFsLiAyMDIzYSkuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NjYuOTY0MjI1NzY5MDQyOTcsImxlZnQiOjQuNTM0MzEzNjc4NzQxNDU1LCJ3aWR0aCI6OTAuNjE0NzYxMzUyNTM5MDYsImhlaWdodCI6MS43NzU1Mjk2MjMwMzE2MTYyfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NjkuMjM2OTA3OTU4OTg0MzgsImxlZnQiOjQuNTM0MzEzNjc4NzQxNDU1LCJ3aWR0aCI6OTAuMDEwMzA3MzEyMDExNzIsImhlaWdodCI6MS43NzU1Mjk2MjMwMzE2MTYyfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NzEuNTA5NTkwMTQ4OTI1NzgsImxlZnQiOjQuNTM0MzEzNjc4NzQxNDU1LCJ3aWR0aCI6ODUuODEyMTc5NTY1NDI5NjksImhlaWdodCI6MS43NzU1Mjk2MjMwMzE2MTYyfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NzMuNzgyMjcyMzM4ODY3MTksImxlZnQiOjQuNTM0MzEzNjc4NzQxNDU1LCJ3aWR0aCI6ODYuMDcwMzEyNSwiaGVpZ2h0IjoxLjc3NTUyOTYyMzAzMTYxNjJ9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo3Ni4wNTQ5NTQ1Mjg4MDg2LCJsZWZ0Ijo0LjUzNDMxMzY3ODc0MTQ1NSwid2lkdGgiOjg5LjYyMzY2NDg1NTk1NzAzLCJoZWlnaHQiOjEuNzc1NTI5NjIzMDMxNjE2Mn19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjc4LjMyNzYyOTA4OTM1NTQ3LCJsZWZ0Ijo0LjUzNDMxMzY3ODc0MTQ1NSwid2lkdGgiOjg5Ljg1OTc1NjQ2OTcyNjU2LCJoZWlnaHQiOjEuNzc1NTI5NjIzMDMxNjE2Mn19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjgwLjYwMDMxODkwODY5MTQsImxlZnQiOjQuNTM0MzEzNjc4NzQxNDU1LCJ3aWR0aCI6ODkuNjQwODM4NjIzMDQ2ODgsImhlaWdodCI6MS43NzU1Mjk2MjMwMzE2MTYyfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6ODIuODcyOTkzNDY5MjM4MjgsImxlZnQiOjQuNTM0MzEzNjc4NzQxNDU1LCJ3aWR0aCI6OTAuODM2NjM5NDA0Mjk2ODgsImhlaWdodCI6NC4wNDgxOTQ4ODUyNTM5MDZ9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo4Ny40MTgzNDI1OTAzMzIwMywibGVmdCI6NC41MzQzMTM2Nzg3NDE0NTUsIndpZHRoIjozMy43OTA5MzU1MTYzNTc0MiwiaGVpZ2h0IjoxLjc3NTUyOTYyMzAzMTYxNjJ9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo5MS4yMDYxNDYyNDAyMzQzOCwibGVmdCI6NC41MzQzMTM2Nzg3NDE0NTUsIndpZHRoIjo4NC42NzUzNzY4OTIwODk4NCwiaGVpZ2h0IjoxLjc3NTUyOTYyMzAzMTYxNjJ9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo5My40Nzg4MjA4MDA3ODEyNSwibGVmdCI6NC41MzQzMTM2Nzg3NDE0NTUsIndpZHRoIjo4OS45ODM4NDg1NzE3NzczNCwiaGVpZ2h0IjoxLjc3NTUyOTYyMzAzMTYxNjJ9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo0Ljc0OTY0MzgwMjY0MjgyMiwibGVmdCI6NC41MzQzMTM2Nzg3NDE0NTUsIndpZHRoIjo4Ny45NTc4NjI4NTQwMDM5LCJoZWlnaHQiOjEuNzc1NTMwNTc2NzA1OTMyNn19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjcuMDIyMzA5MzAzMjgzNjkxLCJsZWZ0Ijo0LjUzNDMxMzY3ODc0MTQ1NSwid2lkdGgiOjkwLjY2NjUxMTUzNTY0NDUzLCJoZWlnaHQiOjEuNzc1NTMwNTc2NzA1OTMyNn19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjIifX0seyJvcmlnaW4iOjQsIndvcmsiOnsidHlwZSI6OCwic3VidHlwZSI6eyJvbmVvZktpbmQiOiJhcnRpY2xlVHlwZSIsImFydGljbGVUeXBlIjoxfSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiVG9tIiwiZmFtaWx5IjoiSmlsYmVydCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJSYW91bOKAkE1hcmllIiwiZmFtaWx5IjoiQ291dHVyZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJCcmlhbiIsImZhbWlseSI6Ikh1c2VyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkthbGV2aSIsImZhbWlseSI6IlNhbG9uZW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIwMDE4LTgxNTgiLCIxNTczLTUxMTciXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMDcvczEwNzUwLTAyMC0wNDQ1Ny14IiwibWFnIjoiMzEwMDQ1OTA1MCIsIm9wZW5hbGV4IjoiVzMxMDA0NTkwNTAifSwidGl0bGUiOiJQcmVmYWNlOiBSZXN0b3JhdGlvbiBvZiBldXRyb3BoaWMgbGFrZXM6IGN1cnJlbnQgcHJhY3RpY2VzIGFuZCBmdXR1cmUgY2hhbGxlbmdlcyIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDA3L3MxMDc1MC0wMjAtMDQ0NTcteCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9saW5rLnNwcmluZ2VyLmNvbS9jb250ZW50L3BkZi8xMC4xMDA3L3MxMDc1MC0wMjAtMDQ0NTcteC5wZGYiLCJpc3N1ZWREYXRlIjoiMjAyMC0xMS0xNiIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDciLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6Ikh5ZHJvYmlvbG9naWEiLCJpc3N1ZSI6MjEsInZvbHVtZSI6ODQ3LCJpbXBhY3RGYWN0b3IiOiIzLjAwIn0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6NDM0MywibGFzdFBhZ2UiOjQzNTd9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiU3ByaW5nZXIgU2NpZW5jZStCdXNpbmVzcyBNZWRpYSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6OTUsImNpdGVkQnlDb3VudCI6NTh9fSwicXVvdGUiOiJTaW1pbGFyIGRlZ3JhZGF0aW9uIG9mIGZyZXNod2F0ZXIgZWNvc3lzdGVtcyBpcyBvYnNlcnZlZCB3b3JsZHdpZGUsIHdpdGggdGhlIHRpbWVsaW5lIG9mIGh1bWFuIHByZXNzdXJlIGRpY3RhdGVkIGJ5IHRyZW5kcyBpbiB1cmJhbml6YXRpb24gYW5kIGludGVuc2l2ZSBhZ3JpY3VsdHVyZSBpbiBkaWZmZXJlbnQgZ2VvZ3JhcGhpYyByZWdpb25zIChlLmcuLCBWZW5uZW1vIGV0IGFsLiwgMjAwOSkuIFRoZSBwcmluY2lwYWwgZHJpdmVyIG9mIGVjb3N5c3RlbSBzdHJlc3MgaW4gbW9zdCBhcXVhdGljIHN5c3RlbXMgaXMgZXV0cm9waGljYXRpb24sIHRoZSBleGNlc3MgZ3Jvd3RoIG9mIGFsZ2FlIGRyaXZlbiBieSBhbnRocm9wb2dlbmljIGlucHV0cyBvZiB0aGUgbWFjcm9udXRyaWVudHMgbml0cm9nZW4gKE4pIGFuZCBwaG9zcGhvcnVzIChQKSAoU21pdGgsIDIwMDM7IEJpcmsgZXQgYWwuLCAyMDIwKS4gTnV0cmllbnRzIGFyZSBlbWl0dGVkIGZyb20gYm90aCBwb2ludCBhbmQgZGlmZnVzZSBzb3VyY2VzIGluIHRoZSBodW1hbiBlbnZpcm9ubWVudCBhbmQgZW50ZXIgYXF1YXRpYyBzeXN0ZW1zIHZpYSBzdXJmYWNlIHJ1bm9mZiwgYXMgd2VsbCBhcyBncm91bmQtd2F0ZXIgYW5kIGF0bW9zcGhlcmljIGRlcG9zaXRpb24gKFBhZXJsLCAxOTk3KS4gRXV0cm9waGljYXRpb24gb2Z0ZW4gbGVhZHMgdG8gdGhlIGRvbWluYW5jZSBvZiBwcmltYXJ5IHByb2R1Y3Rpb24gYnkgY3lhbm9iYWN0ZXJpYSwgaW5jbHVkaW5nIHRveGljIGdlbmVyYSBzdWNoIGFzIERvbGljaG9zcGVybXVtLCBBcGhhbml6b21lbm9uLCBhbmQgTWljcm9jeXN0aXMgKEh1aXNtYW4gZXQgYWwuLCAyMDE4KSwgYXMgd2VsbCBhcyBveHlnZW4gZGVwbGV0aW9uIGFuZCBoYWJpdGF0IGRlZ3JhZGF0aW9uIGluIGxha2VzIChBcmVuZCBldCBhbC4sIDIwMTEpLiBCZWNhdXNlIGxha2VzIHByb3ZpZGUgYSByYW5nZSBvZiBjcnVjaWFsIGVjb3N5c3RlbSBzZXJ2aWNlcyAoV2lsc29uICYgQ2FycGVudGVyLCAxOTk5KSwgdGhlcmUgaXMgYSBncm93aW5nIG5lZWQgZm9yIG1hbmFnZW1lbnQgYXBwcm9hY2hlcyB0aGF0IHJlc3RvcmUgd2F0ZXIgcXVhbGl0eSBpbiBmcmVzaHdhdGVyIHN5c3RlbXMuIFdpdGhpbiB0aGUgZnJhbWV3b3JrIG9mIFVuaXRlZCBOYXRpb25zIFN1c3RhaW5hYmxlIERldmVsb3AtbWVudCBHb2FsIDYgKENsZWFuIFdhdGVyIGFuZCBTYW5pdGF0aW9uKSwgc2V2ZXJhbCBkaXN0aW5jdCB0YXJnZXRzIHJlbGF0ZSB0byB0aGUgaW1wcm92ZW1lbnQgb2Ygd2F0ZXIgcXVhbGl0eSBhbmQgcHJvdGVjdGlvbiBvZiBhcXVhdGljIGVjb3N5c3RlbXMgYnkgMjAzMCAoVW5pdGVkIE5hdGlvbnMsIDIwMTUpLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI4LjI5ODg2NDM2NDYyNDAyMywibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNzE0OTQyOTMyMTI4OSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyOS45OTExMzQ2NDM1NTQ2ODgsImxlZnQiOjguNTQ5MjA1NzgwMDI5Mjk3LCJ3aWR0aCI6MzkuMzY1MTE5OTM0MDgyMDMsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzEuNjgzNDA2ODI5ODMzOTg0LCJsZWZ0Ijo4LjU0OTIwNTc4MDAyOTI5Nywid2lkdGgiOjM5LjM2OTY2MzIzODUyNTM5LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMzLjM2Nzk3MzMyNzYzNjcyLCJsZWZ0Ijo4LjU0OTIwNTc4MDAyOTI5Nywid2lkdGgiOjM5LjM3NTY3OTAxNjExMzI4LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjM1LjA2MDI0OTMyODYxMzI4LCJsZWZ0Ijo4LjU0OTIwNTc4MDAyOTI5Nywid2lkdGgiOjM5LjM3MDk2NDA1MDI5Mjk3LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjM2Ljc1MjUxNzcwMDE5NTMxLCJsZWZ0Ijo4LjU0OTIwODY0MTA1MjI0Niwid2lkdGgiOjM5LjM3Njk2MDc1NDM5NDUzLCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjM4LjQzNzA4ODAxMjY5NTMxLCJsZWZ0Ijo4LjU0OTIwODY0MTA1MjI0Niwid2lkdGgiOjM5LjM2NjU2OTUxOTA0Mjk3LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQwLjEyOTM1NjM4NDI3NzM0NCwibGVmdCI6OC41NDkyMDg2NDEwNTIyNDYsIndpZHRoIjozOS4zNjQzNzk4ODI4MTI1LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQxLjgyMTYyNDc1NTg1OTM3NSwibGVmdCI6OC41NDkyMDg2NDEwNTIyNDYsIndpZHRoIjozOS4zNjU2NzMwNjUxODU1NSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0My41MDYxOTUwNjgzNTkzNzUsImxlZnQiOjguNTQ5Mzg5ODM5MTcyMzYzLCJ3aWR0aCI6MzkuMzcwMzkxODQ1NzAzMTI1LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQ1LjE5ODQ2MzQzOTk0MTQwNiwibGVmdCI6OC41NDk1NzI5NDQ2NDExMTMsIndpZHRoIjozOS4zNjU4NDQ3MjY1NjI1LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQ2Ljg5MDczNTYyNjIyMDcsImxlZnQiOjguNTQ5NTcyOTQ0NjQxMTEzLCJ3aWR0aCI6MzkuMzcyOTU1MzIyMjY1NjI1LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQ4LjU3NTMwMjEyNDAyMzQ0LCJsZWZ0Ijo4LjU0OTU3Mjk0NDY0MTExMywid2lkdGgiOjM5LjM3MjQyMTI2NDY0ODQ0LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjUwLjI2NzU3ODEyNSwibGVmdCI6OC41NDk1NzY3NTkzMzgzNzksIndpZHRoIjozOS4zNzExMjQyNjc1NzgxMjUsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTEuOTU5ODQyNjgxODg0NzY2LCJsZWZ0Ijo4LjU0OTU3Njc1OTMzODM3OSwid2lkdGgiOjM5LjM2NzgzNTk5ODUzNTE1NiwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1My41NzQxMTk1Njc4NzEwOTQsImxlZnQiOjguNTQ5NTc2NzU5MzM4Mzc5LCJ3aWR0aCI6MzkuMzcwNzc3MTMwMTI2OTUsImhlaWdodCI6MS4zMzY4OTU1ODUwNjAxMTk2fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTUuMjY2MzkxNzU0MTUwMzksImxlZnQiOjguNTQ5NzYwODE4NDgxNDQ1LCJ3aWR0aCI6MzkuMzY2NTgwOTYzMTM0NzY2LCJoZWlnaHQiOjEuMzM2ODk1NTg1MDYwMTE5Nn19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjU3LjAyODk1MzU1MjI0NjA5NCwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNjIwMjYyMTQ1OTk2MSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1OC43MTM1MjM4NjQ3NDYwOTQsImxlZnQiOjguNTQ5OTQ0ODc3NjI0NTEyLCJ3aWR0aCI6OS4yOTk5ODIwNzA5MjI4NTIsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NjAuNDA1NzkyMjM2MzI4MTI1LCJsZWZ0IjoxMC42MjI0NTU1OTY5MjM4MjgsIndpZHRoIjozNy4yODc4Njg0OTk3NTU4NiwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2Mi4wOTgwNjA2MDc5MTAxNTYsImxlZnQiOjguNTQ5OTQzOTIzOTUwMTk1LCJ3aWR0aCI6MzkuMzczMzI1MzQ3OTAwMzksImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NjMuNzgyNjMwOTIwNDEwMTU2LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjM5LjM3MzEyNjk4MzY0MjU4LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjY1LjQ3NDkwNjkyMTM4NjcyLCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjM5LjM2NTExOTkzNDA4MjAzLCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjY3LjE2NzE3NTI5Mjk2ODc1LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjM5LjM2OTQ4MDEzMzA1NjY0LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjY4Ljc4MTQ0ODM2NDI1NzgxLCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjM5LjM3NzMzODQwOTQyMzgzLCJoZWlnaHQiOjEuMzM2ODkxNTMxOTQ0Mjc1fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NzAuNTQ0MDEzOTc3MDUwNzgsImxlZnQiOjguNTQ5OTQ0ODc3NjI0NTEyLCJ3aWR0aCI6MzkuMzY5MzA4NDcxNjc5NjksImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NzIuMjM2Mjg5OTc4MDI3MzQsImxlZnQiOjguNTQ5OTQ0ODc3NjI0NTEyLCJ3aWR0aCI6MzkuMzcyMDMyMTY1NTI3MzQ0LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjczLjkyMDg0NTAzMTczODI4LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjM5LjM3MTMwNzM3MzA0Njg3NSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI0MzQ0In19XQ%3D%3D. The dominant cyanobacterial taxa reported from Indian HABs and their principal toxins are summarized in Table 2. This succession cascades through trophic levels zooplankton decline as grazers succumb to toxins, rotifers proliferate, and fish biodiversity plummets by 30-60% amid hypoxia (Shahbaz et al., 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1kIiwiZmFtaWx5IjoiU2hhaGJheiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBaGJhciIsImZhbWlseSI6IkFsYW0ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTW9oYW1tYWQgTWFzcm9vciIsImZhbWlseSI6IlphZmFyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1vaGFtbWVkIEFhc2lmIiwiZmFtaWx5IjoiU3VsYWltYW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW51cG1hIiwiZmFtaWx5IjoiS3VtYXJpIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkcuIEsuIiwiZmFtaWx5IjoiU2hhcm1hIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNoYWhsYSIsImZhbWlseSI6Illhc21pbiJ9fV0sImlkcyI6eyJpc3NuIjpbIjEyMTUtMTYxMCIsIjIwNjEtOTU4OCJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMjQ3OC9vcmh1LTIwMjMtMDAxNiIsIm9wZW5hbGV4IjoiVzQzODk1MTAwMzcifSwidGl0bGUiOiJDb21wYXJpc29uIG9mIGF2aWFuIGRpdmVyc2l0eSBiZXR3ZWVuIG1hbmFnZWQgYW5kIHVubWFuYWdlZCB3ZXRsYW5kcyBpbiBQYXRuYSwgQmloYXIsIEluZGlhIiwiYWJzdHJhY3QiOiJBYnN0cmFjdCBXZSBjb21wYXJlZCB0aGUgYmlyZCBkaXZlcnNpdHkgb2YgYSBtYW5hZ2VkIChSYWpkaGFuaSBKYWxhc2hheSkgYW5kIGFuIHVubWFuYWdlZCB3ZXRsYW5kIChNdXN0YWZhcHVyKSBpbiBQYXRuYSwgQmloYXIsIEluZGlhLiBXZSByZWNvcmRlZCBiaXJkIHNwZWNpZXMgYnkgdXNpbmcgcG9pbnQgY291bnRzIGFuZCBmaXhlZC1yb3V0ZSBtb25pdG9yaW5nLiBBdCBSYWpkaGFuaSBKYWxhc2hheSwgYSB0b3RhbCBvZiA3MyBzcGVjaWVzIHdlcmUgcmVjb3JkZWQsIG9mIHdoaWNoIDE1IHNwZWNpZXMgd2VyZSB3aW50ZXIgdmlzaXRvcnMuIFR3byBzcGVjaWVzIHdlcmUgaW4gdGhlIE5lYXItdGhyZWF0ZW5lZCBjYXRlZ29yeTogRmVycnVnaW5vdXMgRHVjayAoQXl0aHlhIG55cm9jYSkgYW5kIEFsZXhhbmRyaW5lIFBhcmFrZWV0IChQc2l0dGFjdWxhIGV1cGF0cmlhKSAsIHdoaWxlIHRoZSBvdGhlcnMgd2VyZSBvZiBsZWFzdCBjb25jZXJuLiBBdCBNdXN0YWZhcHVyIHdldGxhbmQsIDY3IHNwZWNpZXMgd2VyZSByZWNvcmRlZCB3aXRoIDExIHNwZWNpZXMgYXMgd2ludGVyIHZpc2l0b3JzLiBTaW1wc29u4oCZcyBzcGVjaWVzIGRpdmVyc2l0eSBpbmRleCB3YXMgMC43OCBhdCBSYWpkaGFuaSBKYWxhc2hheSwgYW5kIDAuODEgYXQgTXVzdGFmYXB1ciB3ZXRsYW5kLiBUaGUgbnVtYmVyIG9mIHNwZWNpZXMgb2Ygd2FkZXJzIHdhcyAxNyBhdCBNdXN0YWZhcHVyIHdldGxhbmQgYW5kIDExIGF0IFJhamRoYW5pIEphbGFzaGF5LiBUaGUgdG90YWwgbnVtYmVyIG9mIExlc3NlciBXaGlzdGxpbmctZHVjayAoRGVuZHJvY3lnbmEgamF2YW5pY2EpICwgYSByZXNpZGVudCBiaXJkIHdhcyBhbG1vc3QgZG91YmxlICg1MjYpIGF0IE11c3RhZmFwdXIgd2V0bGFuZCB0aGFuIHRoYXQgYXQgUmFqZGhhbmkgSmFsYXNoYXkgKDIzNCkuIFRoZSBuaXRyYXRlIGxldmVsIHdhcyB0d2ljZSBoaWdoZXIgYW5kIHRoZSBwaG9zcGhhdGUgbGV2ZWwgd2FzIGZvdXIgdGltZXMgaGlnaGVyIGF0IE11c3RhZmFwdXIgd2V0bGFuZCB0aGFuIHRob3NlIGF0IFJhamRoYW5pIEphbGFzaGF5LiBUaGUgZGlmZmVyZW5jZSBpbiBzcGVjaWVzIG51bWJlciBhbmQgY29tcG9zaXRpb24gYmV0d2VlbiB0aGUgdHdvIHdldGxhbmRzIG1heSBiZSBhdHRyaWJ1dGVkIHRvIHRoZSBldXRyb3BoaWNhdGlvbiByZXN1bHRpbmcgZnJvbSBoaWdoIG5pdHJhdGUgYW5kIHBob3NwaGF0ZSBsZXZlbHMgYXQgTXVzdGFmYXB1ciB3ZXRsYW5kLiBVbmZvcnR1bmF0ZWx5LCB0aGUgTXVzdGFmYXB1ciB3ZXRsYW5kIGlzIHVuZGVyIHRocmVhdCBkdWUgdG8gaHVtYW4gaW5mbHVlbmNlcy4gVGhlIHByb3RlY3Rpb24gYW5kIHN1c3RhaW5hYmxlIG1hbmFnZW1lbnQgb2YgbmF0dXJhbCB3ZXRsYW5kcyBpcyByZXF1aXJlZCBmb3Igc2F2aW5nIHRoZSBiaW9kaXZlcnNpdHkgb2YgdGhlIGFyZWEuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjI0Nzgvb3JodS0yMDIzLTAwMTYiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vc2NpZW5kby5jb20vcGRmLzEwLjI0Nzgvb3JodS0yMDIzLTAwMTYiLCJpc3N1ZWREYXRlIjoiMjAyMy0xMi0wMSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6Ik9ybmlzIEh1bmdhcmljYSIsImlzc3VlIjoyLCJ2b2x1bWUiOjMxLCJpbXBhY3RGYWN0b3IiOiIwLjkyIn0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTMsImxhc3RQYWdlIjoyOH19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJIdW5nYXJpYW4gT3JuaXRob2xvZ2ljYWwgYW5kIE5hdHVyZSBDb25zZXJ2YXRpb24gU29jaWV0eSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MjcsImNpdGVkQnlDb3VudCI6MX19LCJxdW90ZSI6Ikhvd2V2ZXIsIGhpZ2ggbGV2ZWxzIG9mIGV1dHJvcGhpY2F0aW9uIG1heSBhbHNvIHRyaWdnZXIgdGhlIHByb2xpZmVyYXRpb24gb2YgZ3JlZW4gYWxnYWUgYW5kICBkZWNsaW5lIG9mIG1hY3JvcGh5dGVzIGxlYWRpbmcgdG8gYSBoaWdoLXR1cmJpZGl0eSBzdGF0ZSBvZiB3YXRlciwgaW4gd2hpY2ggZmV3IHNwZWNpZXMgY2FuICBzdXJ2aXZlIChTY2hlZmZlciBldCBhbC4gMTk5MykuIFRvIGFkZHJlc3MgdGhlIGV1dHJvcGhpY2F0aW9uIGFuZCBleGNlc3MgbnV0cmllbnQgaXNzdWVzIGluICBhcXVhdGljIGVjb3N5c3RlbXMsIHBoeXRvcmVtZWRpYXRpb24gb2ZmZXJzIGEgcHJvbWlzaW5nIHNvbHV0aW9uIChMaXUgZXQgYWwuIDIwMjEpLiBBcXVhdGljICBwbGFudHMgbGlrZSBkdWNrd2VlZHMsIHdhdGVyIGxldHR1Y2UgKFBpc3RpYSBzdHJhdGlvdGVzKSwgYW5kIHdhdGVyIGxpbGllcyBoYXZlIGRlbW9uc3RyYXRlZCAgdGhlaXIgYWJpbGl0eSB0byBjb21iYXQgZXV0cm9waGljYXRpb24gYnkgYWJzb3JiaW5nIGFuZCBhc3NpbWlsYXRpbmcgZXhjZXNzIHBob3NwaG9ydXMgYW5kICBuaXRyb2dlbiAoTmFoYXIgJiBIb3F1ZSAyMDIxKS4gSG93ZXZlciwgdGhlc2UgcGxhbnRzIGNhbiBzb21ldGltZXMgZ3JvdyB1bmNvbnRyb2xsYWJseSwgIHBvdGVudGlhbGx5IGNvdmVyaW5nIHRoZSB3YXRlciBzdXJmYWNlcy4gVG8gYXZvaWQgc3VjaCBzY2VuYXJpb3MsIHJlZ3VsYXIgbW9uaXRvcmluZyBpcyBlc3NlbnRpYWwgKExpdSBldCBhbC4gMjAyMSkuIFByb21vdGluZyBwdWJsaWMgYXdhcmVuZXNzIGlzIHRoZSBjb3JuZXJzdG9uZSBmb3IgbW9iaWxpemluZyBzdXBwb3J0IGFuZCBlbnN1cmluZyBlZmZlY3RpdmUgZXV0cm9waGljYXRpb24gbWFuYWdlbWVudC4gQ29udHJvbGxlZCBlcmFkaWNhdGlvbiBvZiBleGNlc3MgIGZsb3JhIHdoZW4gbmVjZXNzYXJ5IGlzIHZpdGFsIHRvIHByZXZlbnQgb3Zlcmdyb3d0aC4gVGhpcyBpbnRlZ3JhdGVkIGFwcHJvYWNoIGNhbiBlZmZlY3RpdmVseSAgcmVkdWNlIGV1dHJvcGhpY2F0aW9uIGluIHRoZSB3YXRlciBib2RpZXMgKFdhbmcgZXQgYWwuIDIwMjIpLiBFYWNoIHNwZWNpZXMgb2Ygd2V0bGFuZC1kZXBlbmRlbnQgYmlyZCBoYXMgYSB1bmlxdWUgYW5kIGNvbXBsZXggc2V0IG9mIG5lZWRzIGZvciB3ZXRsYW5kIGhhYml0YXRzIHRoYXQgbWFrZXMgaXQgZGlmZmljdWx0IHRvIGdlbmVyYWxpemUgYWJvdXQgaG93IGxvc3Mgb3IgZGVncmFkYXRpb24gb2Ygd2V0bGFuZHMgYWZmZWN0cyBiaXJkIHBvcHVsYXRpb25zLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo3OS4yOTU2MzkwMzgwODU5NCwibGVmdCI6MTAuMzAzMDA2MTcyMTgwMTc2LCJ3aWR0aCI6NzkuOTUyNzI4MjcxNDg0MzgsImhlaWdodCI6MS43MDkxOTc4Nzg4Mzc1ODU0fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjgxLjIxMjUzOTY3Mjg1MTU2LCJsZWZ0IjoxMC4zMDMwMDYxNzIxODAxNzYsIndpZHRoIjo3OS45NTkyODE5MjEzODY3MiwiaGVpZ2h0IjoxLjcwOTE5Nzg3ODgzNzU4NTR9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6ODMuMTI5NDQ3OTM3MDExNzIsImxlZnQiOjEwLjMwMzAwNjE3MjE4MDE3Niwid2lkdGgiOjc5Ljk0MjcyNjEzNTI1MzksImhlaWdodCI6MS43MDkxOTc4Nzg4Mzc1ODU0fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjg1LjA0NjM0ODU3MTc3NzM0LCJsZWZ0IjoxMC4zMDMwMDgwNzk1Mjg4MDksIndpZHRoIjo3OS45Mzk0NTMxMjUsImhlaWdodCI6MS43MDkxOTc4Nzg4Mzc1ODU0fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjg2Ljk2MzI1NjgzNTkzNzUsImxlZnQiOjEwLjMwMzAxMDk0MDU1MTc1OCwid2lkdGgiOjc5Ljk0MTg0ODc1NDg4MjgxLCJoZWlnaHQiOjEuNzA5MTk3ODc4ODM3NTg1NH19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo4OC44ODAxNjUxMDAwOTc2NiwibGVmdCI6MTAuMzAzMDEzODAxNTc0NzA3LCJ3aWR0aCI6NzkuOTM3NSwiaGVpZ2h0IjoxLjcwOTE5Nzg3ODgzNzU4NTR9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6OTAuNzk3MDY1NzM0ODYzMjgsImxlZnQiOjEwLjMwMzAxMzgwMTU3NDcwNywid2lkdGgiOjc5LjkzOTY1MTQ4OTI1NzgxLCJoZWlnaHQiOjEuNzA5MTk3ODc4ODM3NTg1NH19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo5Mi43MTM5NzM5OTkwMjM0NCwibGVmdCI6MTAuMzAzMDEzODAxNTc0NzA3LCJ3aWR0aCI6NzkuOTUwMzI1MDEyMjA3MDMsImhlaWdodCI6MS43MDkxOTc4Nzg4Mzc1ODU0fX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjEyLjIwNzk5MDY0NjM2MjMwNSwibGVmdCI6MTAuMzAzMDI0MjkxOTkyMTg4LCJ3aWR0aCI6NzkuOTUyNTE0NjQ4NDM3NSwiaGVpZ2h0Ijo3LjQ1OTkyMjc5MDUyNzM0NH19LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjoxOS44NzU2MjU2MTAzNTE1NjIsImxlZnQiOjEyLjEyMTMyNDUzOTE4NDU3LCJ3aWR0aCI6NzguMTE5MjI0NTQ4MzM5ODQsImhlaWdodCI6MS43MDkxOTc4Nzg4Mzc1ODU0fX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjIxLjc5MjUzMzg3NDUxMTcyLCJsZWZ0IjoxMC4zMDMwMjkwNjAzNjM3Nywid2lkdGgiOjc5Ljk0OTI0OTI2NzU3ODEyLCJoZWlnaHQiOjEuNzA5MTk3ODc4ODM3NTg1NH19LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjoyMy43MDk0MzY0MTY2MjU5NzcsImxlZnQiOjEwLjMwMzAyOTA2MDM2Mzc3LCJ3aWR0aCI6NzkuOTUyMjcwNTA3ODEyNSwiaGVpZ2h0IjoxLjcwOTE5Nzg3ODgzNzU4NTR9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIyNSJ9fV0%3D. In India, this manifests in lakes like Chilika and Vembanad, where blooms cover 30% of surfaces, reducing transparency to <0.5 m.

Table 2: Dominant Cyanobacterial Species in Indian Lakes

| Species | Toxins Produced |
| --- | --- |
| Microcystis aeruginosa | Microcystins (hepatotoxins) |
| Dolichospermum spp. | Anatoxins, Saxitoxins (neurotoxins) |
| Cylindrospermopsis raciborskii | Cylindrospermopsin (hepatotoxin) |
| Planktothrix spp. | Microcystins (hepatotoxins) |

Chemically, bloom senescence unleashes cascading disturbances: algal decay by heterotrophic bacteria consumes dissolved oxygen, plummeting levels below 2 mg/L and creating anoxic "dead zones" (Sitote & Gebremedhine, 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllhYnNyYSBNZWxhayIsImZhbWlseSI6IlNpdG90ZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNaWNoYWVsIEdpcmltYXkiLCJmYW1pbHkiOiJHZWJyZW1lZGhpbmUifX1dLCJpZHMiOnsiaXNzbiI6W10sImlzYm4iOltdLCJkb2kiOiIxMC4yMDk0NC9wcmVwcmludHMyMDI0MDkuMDcwNC52MSIsIm9wZW5hbGV4IjoiVzQ0MDI0NjU0MTQiLCJleGEiOiJodHRwczovL3d3dy5wcmVwcmludHMub3JnL21hbnVzY3JpcHQvMjAyNDA5LjA3MDQifSwidGl0bGUiOiJDb21wcmVoZW5zaXZlIFJldmlldyBvZiBFdXRyb3BoaWNhdGlvbiBpbiBGcmVzaHdhdGVyIEVjb3N5c3RlbXM6IENhdXNlcywgRWZmZWN0cywgQXNzZXNzbWVudCwgYW5kIE1hbmFnZW1lbnQgU3RyYXRlZ2llcyIsImFic3RyYWN0IjoiVGhpcyByZXZpZXcgZXhhbWluZXMgZXV0cm9waGljYXRpb24gaW4gZnJlc2h3YXRlciBlY29zeXN0ZW1zLCBhIHdpZGVzcHJlYWQgZW52aXJvbm1lbnRhbCBpc3N1ZSBzdGVtbWluZyBmcm9tIGV4Y2Vzc2l2ZSBudXRyaWVudCBpbnB1dCwgbGFyZ2VseSBkdWUgdG8gaHVtYW4gYWN0aXZpdGllcy4gVGhlIHN0dWR5IGV4cGxvcmVzIHRoZSBvcmlnaW5zLCBjb25zZXF1ZW5jZXMsIGV2YWx1YXRpb24gdGVjaG5pcXVlcywgYW5kIGNvbnRyb2wgbWV0aG9kcyBmb3IgZXV0cm9waGljYXRpb24uIFByaW1hcnkgY2F1c2VzIGluY2x1ZGUgYWdyaWN1bHR1cmFsIGRpc2NoYXJnZSwgdXJiYW4gZGV2ZWxvcG1lbnQsIGFuZCBjbGltYXRlIHNoaWZ0cy4gVGhlIGVmZmVjdHMgYXJlIGZhci1yZWFjaGluZywgaW1wYWN0aW5nIGJpb2RpdmVyc2l0eSwgd2F0ZXIgcXVhbGl0eSwgYW5kIHNvY2lvLWVjb25vbWljIGZhY3RvcnMuIFRoZSBhc3Nlc3NtZW50IHV0aWxpemVzIGEgY29tYmluYXRpb24gb2YgY2hlbWljYWwsIHBoeXNpY2FsLCBhbmQgYmlvbG9naWNhbCBtYXJrZXJzLCB3aXRoIGN1dHRpbmctZWRnZSB0ZWNobm9sb2dpZXMgbGlrZSByZW1vdGUgc2Vuc2luZyBpbXByb3ZpbmcgbW9uaXRvcmluZyBlZmZvcnRzLiBNaXRpZ2F0aW9uIHN0cmF0ZWdpZXMgYXJlIGRpdmVyc2UsIGluY29ycG9yYXRpbmcgcHJldmVudGF0aXZlIGFjdGlvbnMgKHN1Y2ggYXMgbnV0cmllbnQgbGltaXRhdGlvbiBhbmQgbGFuZCBtYW5hZ2VtZW50KSwgY29ycmVjdGl2ZSBtZWFzdXJlcyAoaW5jbHVkaW5nIGJpby1tYW5pcHVsYXRpb24gYW5kIGNoZW1pY2FsIGFwcGxpY2F0aW9ucyksIGFuZCByZWd1bGF0b3J5IGluaXRpYXRpdmVzIChsaWtlIHdhdGVyIHF1YWxpdHkgZ3VpZGVsaW5lcyBhbmQgbnV0cmllbnQgZXhjaGFuZ2Ugc2NoZW1lcykuIFdoaWxlIGNhc2Ugc3R1ZGllcyBkZW1vbnN0cmF0ZSBzdWNjZXNzZnVsIGludGVydmVudGlvbnMsIHRoZXkgYWxzbyByZXZlYWwgb25nb2luZyBjaGFsbGVuZ2VzLCBwYXJ0aWN1bGFybHkgZWNvc3lzdGVtIHVucHJlZGljdGFiaWxpdHkgYW5kIGNsaW1hdGUgY2hhbmdlIGluZmx1ZW5jZXMuIFRoZSByZXNlYXJjaCBoaWdobGlnaHRzIHRoZSBwcm9taXNlIG9mIGVtZXJnaW5nIHRlY2hub2xvZ2llcywgbm90YWJseSBtYWNoaW5lIGxlYXJuaW5nLCBpbiBlbmhhbmNpbmcgbWFuYWdlbWVudCBwcmFjdGljZXMuIEtub3dsZWRnZSBnYXBzIHBlcnNpc3QsIGVzcGVjaWFsbHkgcmVnYXJkaW5nIGV1dHJvcGhpY2F0aW9uLWJpb2RpdmVyc2l0eSBkeW5hbWljcyBpbiB0cm9waWNhbCBlbnZpcm9ubWVudHMgYW5kIHRoZSBzb2Npby1lY29ub21pYyByYW1pZmljYXRpb25zIG9mIGludGVydmVudGlvbnMuIFRoZSBzdHVkeSBlbXBoYXNpemVzIHRoZSBpbXBvcnRhbmNlIG9mIGZsZXhpYmxlLCBjb21wcmVoZW5zaXZlIGFwcHJvYWNoZXMgdGhhdCBibGVuZCBzY2llbnRpZmljIGluc2lnaHRzIHdpdGggcHJhY3RpY2FsIHNvbHV0aW9ucyBhbmQgc3Ryb25nIHBvbGljeSBzdHJ1Y3R1cmVzIHRvIHRhY2tsZSB0aGlzIGludHJpY2F0ZSBlbnZpcm9ubWVudGFsIGNoYWxsZW5nZSBpbiBhbiBlcmEgb2YgY2xpbWF0ZSBjaGFuZ2UuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjIwOTQ0L3ByZXByaW50czIwMjQwOS4wNzA0LnYxIiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5wcmVwcmludHMub3JnL2Zyb250ZW5kL21hbnVzY3JpcHQvZTA4ZTViNTY1MzNlMmFhZDc0N2ZkOTQzNWU2N2NhNzMvZG93bmxvYWRfcHViIiwiaXNzdWVkRGF0ZSI6IjIwMjQtMDktMTAiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTEyIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJQcmVwcmludHMub3JnIiwiaW1wYWN0RmFjdG9yIjoiMS4xNSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MCwiY2l0ZWRCeUNvdW50Ijo1fX0sInF1b3RlIjoiVGhlIHN0dWR5IGVtcGhhc2l6ZXMgdGhlIGltcG9ydGFuY2Ugb2YgZmxleGlibGUsIGNvbXByZWhlbnNpdmUgYXBwcm9hY2hlcyB0aGF0IGJsZW5kIHNjaWVudGlmaWMgaW5zaWdodHMgd2l0aCBwcmFjdGljYWwgc29sdXRpb25zIGFuZCBzdHJvbmcgcG9saWN5IHN0cnVjdHVyZXMgdG8gdGFja2xlIHRoaXMgaW50cmljYXRlIGVudmlyb25tZW50YWwgY2hhbGxlbmdlIGluIGFuIGVyYSBvZiBjbGltYXRlIGNoYW5nZS4gRXV0cm9waGljYXRpb24sIGNhdXNlZCBieSBleGNlc3NpdmUgbnV0cmllbnQgZW5yaWNobWVudCBpbiB3YXRlciBib2RpZXMsIGlzIGEgc2lnbmlmaWNhbnQgZW52aXJvbm1lbnRhbCBjaGFsbGVuZ2UgZm9yIGZyZXNod2F0ZXIgZWNvc3lzdGVtcyBvbiBhIGdsb2JhbCBzY2FsZS4gUHJpbWFyaWx5IGRyaXZlbiBieSBuaXRyb2dlbiBhbmQgcGhvc3Bob3J1cyBpbnB1dHMsIHRoaXMgcHJvY2VzcyByZXN1bHRzIGluIHRoZSBvdmVycHJvZHVjdGlvbiBvZiBhbGdhZSBhbmQgYXF1YXRpYyBwbGFudHMsIGxlYWRpbmcgdG8gYSBjYXNjYWRlIG9mIGVjb2xvZ2ljYWwgZGlzdHVyYmFuY2VzLCBpbmNsdWRpbmcgb3h5Z2VuIGRlcGxldGlvbiAoaHlwb3hpYSksIGxvc3Mgb2YgYmlvZGl2ZXJzaXR5LCBhbmQgc2V2ZXJlIHdhdGVyIHF1YWxpdHkgZGVncmFkYXRpb24gKEtha2FkZSBldCBhbC4sIDIwMjE7IE1lZXJob2ZmIGV0IGFsLiwgMjAyMjsgV3VydHNiYXVnaCBldCBhbC4sIDIwMTkpLiJ9XQ%3D%3D. Anaerobic respiration releases phosphates from sediments (internal loading), ammonium, and sulfides, perpetuating a vicious eutrophic cycle with biochemical oxygen demand >20 mg/L 10x CPCB limits (Kumar & Mahajan, 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlBhd2FuIiwiZmFtaWx5IjoiS3VtYXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW1icmlzaCBLdW1hciIsImZhbWlseSI6Ik1haGFqYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyNTIzLTM5NjMiLCIyNTIzLTM5NzEiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMDcvczQyNDUyLTAyMC0zMDgyLTgiLCJtYWciOiIzMDM1OTYwMjI4Iiwib3BlbmFsZXgiOiJXMzAzNTk2MDIyOCJ9LCJ0aXRsZSI6IlRyb3BoaWMgc3RhdHVzIGFuZCBpdHMgcmVndWxhdGluZyBmYWN0b3IgZGV0ZXJtaW5hdGlvbiBhdCB0aGUgUmV3YWxzYXIgTGFrZSwgbm9ydGh3ZXN0IEhpbWFsYXlhIChIUCksIEluZGlhLCBiYXNlZCBvbiBzZWxlY3RlZCBwYXJhbWV0ZXJzIGFuZCBtdWx0aXZhcmlhdGUgc3RhdGlzdGljYWwgYW5hbHlzaXMiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAwNy9zNDI0NTItMDIwLTMwODItOCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9saW5rLnNwcmluZ2VyLmNvbS9jb250ZW50L3BkZi8xMC4xMDA3L3M0MjQ1Mi0wMjAtMzA4Mi04LnBkZiIsImlzc3VlZERhdGUiOiIyMDIwLTA2LTIyIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiU04gQXBwbGllZCBTY2llbmNlcyIsImlzc3VlIjo3LCJ2b2x1bWUiOjIsImltcGFjdEZhY3RvciI6IjcuMzUifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiU3ByaW5nZXIgTmF0dXJlIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo0MCwiY2l0ZWRCeUNvdW50IjoxOX19LCJxdW90ZSI6IkNoYW5naW5nIHF1YWxpdHkgaGFzIGNyZWF0ZWQgdGhlIHdhdGVyIHNjYXJjaXR5IGFuZCAgYSBodWdlIG1hc3Mgb2YgcG9wdWxhdGlvbiB0aHVzIGJlZW4gZGVwcml2ZWQgZnJvbSB1c2luZyAgdGhlIGZyZXNod2F0ZXIgZm9yIHZhcmlvdXMgcHVycG9zZXMgWzMsIDRdLiBIb3dldmVyLCB0aGUgIG9ibm94aW91cyBtYXRlcmlhbHMgdGhhdCBzdGF5IGluIHdhdGVyIGJvZGllcyBmb3IgYSBsb25nLXRpbWUgcGVyaW9kIGJyaW5nIGNoYW5nZXMgaW4gdGhlIHBoeXNpb2NoZW1pY2FsIGNoYXItYWN0ZXJpc3RpY3MgYXMgd2VsbCBhcyB0aGUgbmF0dXJhbCBzY2VuYXJpbyBvZiB0aGUgbGFrZXMgWzVdLiAgVGhlIEhpbWFsYXlhbiBsYWtlcyBhcmUgdW5kZXIgc3RyZXNzIGR1ZSB0byBtaXNtYW5hZ2VkIGh1bWFuIGFjdGl2aXRpZXMsIHdoaWNoIGluY3JlYXNlIG51dHJpZW50cyBhbmQgcG9sbHV0aW9uIGluIHRlcm1zIG9mIGhlYXZ5IG1ldGFscyB0b3hpY2l0eSBhbmQgYmFjdGVyaWFsIGNvbnRhbWluYXRpb24gIFs24oCTOF0uIFRoZSBtYW5hZ2VtZW50IGFuZCBwcmVzZXJ2YXRpb24gb2YgdGhlIGxha2VzIGF0ICBwcmVzZW50IGhhdmUgYmVjb21lIGEgZ3JlYXQgY2hhbGxlbmdlIGZvciBlbnZpcm9ubWVudGFsLWlzdHMgYW5kIG1hbmFnZXJzIFs1LCA5XS4gVGhlIGV1dHJvcGhpY2F0aW9uIGlzIGFsc28gYSBtYWluIHByb2JsZW0gb2YgdGhlICBmcmVzaHdhdGVyIGxha2VzLCBhbmQgaXQgaXMgc3BlY2lmaWVkIGZvciB0aGUgbGFrZXMgaGF2aW5nICBudXRyaWVudCBlbnJpY2htZW50LiBJdCBjYXVzZXMgZXhjZXNzaXZlIGdyb3d0aCBvZiBhbGdhZSAgYW5kIGlzIGNvbnNpZGVyZWQgdG8gYmUgcmVzcG9uc2libGUgZm9yIGRlZ3JhZGluZyB0aGUgIHF1YWxpdGF0aXZlIG5hdHVyZSBvZiB0aGUgbGFrZXMgWzUsIDEwXS4gVGhlIGV1dHJvcGhpY2F0aW9uICBpcyBmb2xsb3dlZCBieSB0aGUgYWxnYWwgYmxvb20sIHdoaWNoIGdlbmVyYXRlcyB0b3hpbiwgIHdvcnNlbnMgd2F0ZXIgcXVhbGl0eSwgZGVwbGV0ZXMgZGlzc29sdmVkIG94eWdlbiAoRE8pICogUGF3YW4gS3VtYXIsIHBhd2FuLmV2c0BnbWFpbC5jb20gfCAxRGVwYXJ0bWVudCBvZiBFbnZpcm9ubWVudGFsIFNjaWVuY2VzLCBDZW50cmFsIFVuaXZlcnNpdHkgb2YgSGltYWNoYWwgUHJhZGVzaCwgRGhhcmFtc2hhbGEgMTc2MjE1LCBJbmRpYS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2OC41Nzg0NjA2OTMzNTkzOCwibGVmdCI6OC41NzE0MTg3NjIyMDcwMzEsIndpZHRoIjo0MC4zMjc1MTQ2NDg0Mzc1LCJoZWlnaHQiOjEuNTE5ODU1MjYwODQ4OTk5fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NzAuMTU4OTk2NTgyMDMxMjUsImxlZnQiOjguNTY5NzM4Mzg4MDYxNTIzLCJ3aWR0aCI6NDAuMzU0NzI0ODg0MDMzMiwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjcxLjczOTU0NzcyOTQ5MjE5LCJsZWZ0Ijo4LjU2OTczODM4ODA2MTUyMywid2lkdGgiOjQwLjM0OTY5MzI5ODMzOTg0NCwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjczLjMyMDA5MTI0NzU1ODYsImxlZnQiOjguNTY4MDU5OTIxMjY0NjQ4LCJ3aWR0aCI6NDAuMDIwMDg4MTk1ODAwNzgsImhlaWdodCI6MS41MTk4NTUyNjA4NDg5OTl9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo3NC45MDA2NDIzOTUwMTk1MywibGVmdCI6OC41NjgwNTk5MjEyNjQ2NDgsIndpZHRoIjozOS45OTk5Mjc1MjA3NTE5NSwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjc2LjQ4MTE4NTkxMzA4NTk0LCJsZWZ0Ijo4LjU2ODA1NzA2MDI0MTcsIndpZHRoIjo0MC4zNTcwNzQ3Mzc1NDg4MywiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjc4LjA2MTcyOTQzMTE1MjM0LCJsZWZ0Ijo4LjU2OTczNjQ4MDcxMjg5LCJ3aWR0aCI6NDAuMzUzNzEzOTg5MjU3ODEsImhlaWdodCI6MS41MTk4NTUyNjA4NDg5OTl9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2MC42NzU3Mjc4NDQyMzgyOCwibGVmdCI6NTEuNDI3MTY1OTg1MTA3NDIsIndpZHRoIjo0MC4zNzg1NjY3NDE5NDMzNiwiaGVpZ2h0IjoxOC45MDU4NTUxNzg4MzMwMDh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo4MS45MjczMzc2NDY0ODQzOCwibGVmdCI6OC40MDE0OTc4NDA4ODEzNDgsIndpZHRoIjo3My45NTQ2OTY2NTUyNzM0NCwiaGVpZ2h0IjoyLjY3ODc2MjY3NDMzMTY2NX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEifX1d. pH fluctuations from daytime photosynthesis (CO₂ drawdown) to nocturnal respiration exacerbate toxicity, with cyanotoxins like microcystins accumulating in biomass, rendering water unsafe (Jilbert et al., 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRvbSIsImZhbWlseSI6IkppbGJlcnQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUmFvdWzigJBNYXJpZSIsImZhbWlseSI6IkNvdXR1cmUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQnJpYW4iLCJmYW1pbHkiOiJIdXNlciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLYWxldmkiLCJmYW1pbHkiOiJTYWxvbmVuIn19XSwiaWRzIjp7Imlzc24iOlsiMDAxOC04MTU4IiwiMTU3My01MTE3Il0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDA3L3MxMDc1MC0wMjAtMDQ0NTcteCIsIm1hZyI6IjMxMDA0NTkwNTAiLCJvcGVuYWxleCI6IlczMTAwNDU5MDUwIn0sInRpdGxlIjoiUHJlZmFjZTogUmVzdG9yYXRpb24gb2YgZXV0cm9waGljIGxha2VzOiBjdXJyZW50IHByYWN0aWNlcyBhbmQgZnV0dXJlIGNoYWxsZW5nZXMiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAwNy9zMTA3NTAtMDIwLTA0NDU3LXgiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vbGluay5zcHJpbmdlci5jb20vY29udGVudC9wZGYvMTAuMTAwNy9zMTA3NTAtMDIwLTA0NDU3LXgucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjAtMTEtMTYiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA3IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJIeWRyb2Jpb2xvZ2lhIiwiaXNzdWUiOjIxLCJ2b2x1bWUiOjg0NywiaW1wYWN0RmFjdG9yIjoiMy4wMCJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjQzNDMsImxhc3RQYWdlIjo0MzU3fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlNwcmluZ2VyIFNjaWVuY2UrQnVzaW5lc3MgTWVkaWEifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjk1LCJjaXRlZEJ5Q291bnQiOjU4fX0sInF1b3RlIjoiU2ltaWxhciBkZWdyYWRhdGlvbiBvZiBmcmVzaHdhdGVyIGVjb3N5c3RlbXMgaXMgb2JzZXJ2ZWQgd29ybGR3aWRlLCB3aXRoIHRoZSB0aW1lbGluZSBvZiBodW1hbiBwcmVzc3VyZSBkaWN0YXRlZCBieSB0cmVuZHMgaW4gdXJiYW5pemF0aW9uIGFuZCBpbnRlbnNpdmUgYWdyaWN1bHR1cmUgaW4gZGlmZmVyZW50IGdlb2dyYXBoaWMgcmVnaW9ucyAoZS5nLiwgVmVubmVtbyBldCBhbC4sIDIwMDkpLiBUaGUgcHJpbmNpcGFsIGRyaXZlciBvZiBlY29zeXN0ZW0gc3RyZXNzIGluIG1vc3QgYXF1YXRpYyBzeXN0ZW1zIGlzIGV1dHJvcGhpY2F0aW9uLCB0aGUgZXhjZXNzIGdyb3d0aCBvZiBhbGdhZSBkcml2ZW4gYnkgYW50aHJvcG9nZW5pYyBpbnB1dHMgb2YgdGhlIG1hY3JvbnV0cmllbnRzIG5pdHJvZ2VuIChOKSBhbmQgcGhvc3Bob3J1cyAoUCkgKFNtaXRoLCAyMDAzOyBCaXJrIGV0IGFsLiwgMjAyMCkuIE51dHJpZW50cyBhcmUgZW1pdHRlZCBmcm9tIGJvdGggcG9pbnQgYW5kIGRpZmZ1c2Ugc291cmNlcyBpbiB0aGUgaHVtYW4gZW52aXJvbm1lbnQgYW5kIGVudGVyIGFxdWF0aWMgc3lzdGVtcyB2aWEgc3VyZmFjZSBydW5vZmYsIGFzIHdlbGwgYXMgZ3JvdW5kLXdhdGVyIGFuZCBhdG1vc3BoZXJpYyBkZXBvc2l0aW9uIChQYWVybCwgMTk5NykuIEV1dHJvcGhpY2F0aW9uIG9mdGVuIGxlYWRzIHRvIHRoZSBkb21pbmFuY2Ugb2YgcHJpbWFyeSBwcm9kdWN0aW9uIGJ5IGN5YW5vYmFjdGVyaWEsIGluY2x1ZGluZyB0b3hpYyBnZW5lcmEgc3VjaCBhcyBEb2xpY2hvc3Blcm11bSwgQXBoYW5pem9tZW5vbiwgYW5kIE1pY3JvY3lzdGlzIChIdWlzbWFuIGV0IGFsLiwgMjAxOCksIGFzIHdlbGwgYXMgb3h5Z2VuIGRlcGxldGlvbiBhbmQgaGFiaXRhdCBkZWdyYWRhdGlvbiBpbiBsYWtlcyAoQXJlbmQgZXQgYWwuLCAyMDExKS4gQmVjYXVzZSBsYWtlcyBwcm92aWRlIGEgcmFuZ2Ugb2YgY3J1Y2lhbCBlY29zeXN0ZW0gc2VydmljZXMgKFdpbHNvbiAmIENhcnBlbnRlciwgMTk5OSksIHRoZXJlIGlzIGEgZ3Jvd2luZyBuZWVkIGZvciBtYW5hZ2VtZW50IGFwcHJvYWNoZXMgdGhhdCByZXN0b3JlIHdhdGVyIHF1YWxpdHkgaW4gZnJlc2h3YXRlciBzeXN0ZW1zLiBXaXRoaW4gdGhlIGZyYW1ld29yayBvZiBVbml0ZWQgTmF0aW9ucyBTdXN0YWluYWJsZSBEZXZlbG9wLW1lbnQgR29hbCA2IChDbGVhbiBXYXRlciBhbmQgU2FuaXRhdGlvbiksIHNldmVyYWwgZGlzdGluY3QgdGFyZ2V0cyByZWxhdGUgdG8gdGhlIGltcHJvdmVtZW50IG9mIHdhdGVyIHF1YWxpdHkgYW5kIHByb3RlY3Rpb24gb2YgYXF1YXRpYyBlY29zeXN0ZW1zIGJ5IDIwMzAgKFVuaXRlZCBOYXRpb25zLCAyMDE1KS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyOC4yOTg4NjQzNjQ2MjQwMjMsImxlZnQiOjguNTQ5MjA1NzgwMDI5Mjk3LCJ3aWR0aCI6MzkuMzcxNDk0MjkzMjEyODksImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MjkuOTkxMTM0NjQzNTU0Njg4LCJsZWZ0Ijo4LjU0OTIwNTc4MDAyOTI5Nywid2lkdGgiOjM5LjM2NTExOTkzNDA4MjAzLCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMxLjY4MzQwNjgyOTgzMzk4NCwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNjk2NjMyMzg1MjUzOSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozMy4zNjc5NzMzMjc2MzY3MiwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNzU2NzkwMTYxMTMyOCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNS4wNjAyNDkzMjg2MTMyOCwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNzA5NjQwNTAyOTI5NywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNi43NTI1MTc3MDAxOTUzMSwibGVmdCI6OC41NDkyMDg2NDEwNTIyNDYsIndpZHRoIjozOS4zNzY5NjA3NTQzOTQ1MywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC40MzcwODgwMTI2OTUzMSwibGVmdCI6OC41NDkyMDg2NDEwNTIyNDYsIndpZHRoIjozOS4zNjY1Njk1MTkwNDI5NywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0MC4xMjkzNTYzODQyNzczNDQsImxlZnQiOjguNTQ5MjA4NjQxMDUyMjQ2LCJ3aWR0aCI6MzkuMzY0Mzc5ODgyODEyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0MS44MjE2MjQ3NTU4NTkzNzUsImxlZnQiOjguNTQ5MjA4NjQxMDUyMjQ2LCJ3aWR0aCI6MzkuMzY1NjczMDY1MTg1NTUsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDMuNTA2MTk1MDY4MzU5Mzc1LCJsZWZ0Ijo4LjU0OTM4OTgzOTE3MjM2Mywid2lkdGgiOjM5LjM3MDM5MTg0NTcwMzEyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0NS4xOTg0NjM0Mzk5NDE0MDYsImxlZnQiOjguNTQ5NTcyOTQ0NjQxMTEzLCJ3aWR0aCI6MzkuMzY1ODQ0NzI2NTYyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ni44OTA3MzU2MjYyMjA3LCJsZWZ0Ijo4LjU0OTU3Mjk0NDY0MTExMywid2lkdGgiOjM5LjM3Mjk1NTMyMjI2NTYyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0OC41NzUzMDIxMjQwMjM0NCwibGVmdCI6OC41NDk1NzI5NDQ2NDExMTMsIndpZHRoIjozOS4zNzI0MjEyNjQ2NDg0NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1MC4yNjc1NzgxMjUsImxlZnQiOjguNTQ5NTc2NzU5MzM4Mzc5LCJ3aWR0aCI6MzkuMzcxMTI0MjY3NTc4MTI1LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjUxLjk1OTg0MjY4MTg4NDc2NiwibGVmdCI6OC41NDk1NzY3NTkzMzgzNzksIndpZHRoIjozOS4zNjc4MzU5OTg1MzUxNTYsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTMuNTc0MTE5NTY3ODcxMDk0LCJsZWZ0Ijo4LjU0OTU3Njc1OTMzODM3OSwid2lkdGgiOjM5LjM3MDc3NzEzMDEyNjk1LCJoZWlnaHQiOjEuMzM2ODk1NTg1MDYwMTE5Nn19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjU1LjI2NjM5MTc1NDE1MDM5LCJsZWZ0Ijo4LjU0OTc2MDgxODQ4MTQ0NSwid2lkdGgiOjM5LjM2NjU4MDk2MzEzNDc2NiwiaGVpZ2h0IjoxLjMzNjg5NTU4NTA2MDExOTZ9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1Ny4wMjg5NTM1NTIyNDYwOTQsImxlZnQiOjguNTQ5OTQ0ODc3NjI0NTEyLCJ3aWR0aCI6MzkuMzYyMDI2MjE0NTk5NjEsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTguNzEzNTIzODY0NzQ2MDk0LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjkuMjk5OTgyMDcwOTIyODUyLCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjYwLjQwNTc5MjIzNjMyODEyNSwibGVmdCI6MTAuNjIyNDU1NTk2OTIzODI4LCJ3aWR0aCI6MzcuMjg3ODY4NDk5NzU1ODYsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NjIuMDk4MDYwNjA3OTEwMTU2LCJsZWZ0Ijo4LjU0OTk0MzkyMzk1MDE5NSwid2lkdGgiOjM5LjM3MzMyNTM0NzkwMDM5LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjYzLjc4MjYzMDkyMDQxMDE1NiwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzMxMjY5ODM2NDI1OCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2NS40NzQ5MDY5MjEzODY3MiwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNjUxMTk5MzQwODIwMywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2Ny4xNjcxNzUyOTI5Njg3NSwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNjk0ODAxMzMwNTY2NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2OC43ODE0NDgzNjQyNTc4MSwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzczMzg0MDk0MjM4MywiaGVpZ2h0IjoxLjMzNjg5MTUzMTk0NDI3NX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjcwLjU0NDAxMzk3NzA1MDc4LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjM5LjM2OTMwODQ3MTY3OTY5LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjcyLjIzNjI4OTk3ODAyNzM0LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjM5LjM3MjAzMjE2NTUyNzM0NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo3My45MjA4NDUwMzE3MzgyOCwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzEzMDczNzMwNDY4NzUsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNDM0NCJ9fV0%3D. High turbidity from suspended algae further stifles macrophytes, natural oxygenators, replaced by invasives like Eichhornia crassipes, amplifying anoxia (Çelekli̇, 2024; Shahbaz et al., 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1kIiwiZmFtaWx5IjoiU2hhaGJheiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBaGJhciIsImZhbWlseSI6IkFsYW0ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTW9oYW1tYWQgTWFzcm9vciIsImZhbWlseSI6IlphZmFyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1vaGFtbWVkIEFhc2lmIiwiZmFtaWx5IjoiU3VsYWltYW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW51cG1hIiwiZmFtaWx5IjoiS3VtYXJpIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkcuIEsuIiwiZmFtaWx5IjoiU2hhcm1hIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNoYWhsYSIsImZhbWlseSI6Illhc21pbiJ9fV0sImlkcyI6eyJpc3NuIjpbIjEyMTUtMTYxMCIsIjIwNjEtOTU4OCJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMjQ3OC9vcmh1LTIwMjMtMDAxNiIsIm9wZW5hbGV4IjoiVzQzODk1MTAwMzcifSwidGl0bGUiOiJDb21wYXJpc29uIG9mIGF2aWFuIGRpdmVyc2l0eSBiZXR3ZWVuIG1hbmFnZWQgYW5kIHVubWFuYWdlZCB3ZXRsYW5kcyBpbiBQYXRuYSwgQmloYXIsIEluZGlhIiwiYWJzdHJhY3QiOiJBYnN0cmFjdCBXZSBjb21wYXJlZCB0aGUgYmlyZCBkaXZlcnNpdHkgb2YgYSBtYW5hZ2VkIChSYWpkaGFuaSBKYWxhc2hheSkgYW5kIGFuIHVubWFuYWdlZCB3ZXRsYW5kIChNdXN0YWZhcHVyKSBpbiBQYXRuYSwgQmloYXIsIEluZGlhLiBXZSByZWNvcmRlZCBiaXJkIHNwZWNpZXMgYnkgdXNpbmcgcG9pbnQgY291bnRzIGFuZCBmaXhlZC1yb3V0ZSBtb25pdG9yaW5nLiBBdCBSYWpkaGFuaSBKYWxhc2hheSwgYSB0b3RhbCBvZiA3MyBzcGVjaWVzIHdlcmUgcmVjb3JkZWQsIG9mIHdoaWNoIDE1IHNwZWNpZXMgd2VyZSB3aW50ZXIgdmlzaXRvcnMuIFR3byBzcGVjaWVzIHdlcmUgaW4gdGhlIE5lYXItdGhyZWF0ZW5lZCBjYXRlZ29yeTogRmVycnVnaW5vdXMgRHVjayAoQXl0aHlhIG55cm9jYSkgYW5kIEFsZXhhbmRyaW5lIFBhcmFrZWV0IChQc2l0dGFjdWxhIGV1cGF0cmlhKSAsIHdoaWxlIHRoZSBvdGhlcnMgd2VyZSBvZiBsZWFzdCBjb25jZXJuLiBBdCBNdXN0YWZhcHVyIHdldGxhbmQsIDY3IHNwZWNpZXMgd2VyZSByZWNvcmRlZCB3aXRoIDExIHNwZWNpZXMgYXMgd2ludGVyIHZpc2l0b3JzLiBTaW1wc29u4oCZcyBzcGVjaWVzIGRpdmVyc2l0eSBpbmRleCB3YXMgMC43OCBhdCBSYWpkaGFuaSBKYWxhc2hheSwgYW5kIDAuODEgYXQgTXVzdGFmYXB1ciB3ZXRsYW5kLiBUaGUgbnVtYmVyIG9mIHNwZWNpZXMgb2Ygd2FkZXJzIHdhcyAxNyBhdCBNdXN0YWZhcHVyIHdldGxhbmQgYW5kIDExIGF0IFJhamRoYW5pIEphbGFzaGF5LiBUaGUgdG90YWwgbnVtYmVyIG9mIExlc3NlciBXaGlzdGxpbmctZHVjayAoRGVuZHJvY3lnbmEgamF2YW5pY2EpICwgYSByZXNpZGVudCBiaXJkIHdhcyBhbG1vc3QgZG91YmxlICg1MjYpIGF0IE11c3RhZmFwdXIgd2V0bGFuZCB0aGFuIHRoYXQgYXQgUmFqZGhhbmkgSmFsYXNoYXkgKDIzNCkuIFRoZSBuaXRyYXRlIGxldmVsIHdhcyB0d2ljZSBoaWdoZXIgYW5kIHRoZSBwaG9zcGhhdGUgbGV2ZWwgd2FzIGZvdXIgdGltZXMgaGlnaGVyIGF0IE11c3RhZmFwdXIgd2V0bGFuZCB0aGFuIHRob3NlIGF0IFJhamRoYW5pIEphbGFzaGF5LiBUaGUgZGlmZmVyZW5jZSBpbiBzcGVjaWVzIG51bWJlciBhbmQgY29tcG9zaXRpb24gYmV0d2VlbiB0aGUgdHdvIHdldGxhbmRzIG1heSBiZSBhdHRyaWJ1dGVkIHRvIHRoZSBldXRyb3BoaWNhdGlvbiByZXN1bHRpbmcgZnJvbSBoaWdoIG5pdHJhdGUgYW5kIHBob3NwaGF0ZSBsZXZlbHMgYXQgTXVzdGFmYXB1ciB3ZXRsYW5kLiBVbmZvcnR1bmF0ZWx5LCB0aGUgTXVzdGFmYXB1ciB3ZXRsYW5kIGlzIHVuZGVyIHRocmVhdCBkdWUgdG8gaHVtYW4gaW5mbHVlbmNlcy4gVGhlIHByb3RlY3Rpb24gYW5kIHN1c3RhaW5hYmxlIG1hbmFnZW1lbnQgb2YgbmF0dXJhbCB3ZXRsYW5kcyBpcyByZXF1aXJlZCBmb3Igc2F2aW5nIHRoZSBiaW9kaXZlcnNpdHkgb2YgdGhlIGFyZWEuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjI0Nzgvb3JodS0yMDIzLTAwMTYiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vc2NpZW5kby5jb20vcGRmLzEwLjI0Nzgvb3JodS0yMDIzLTAwMTYiLCJpc3N1ZWREYXRlIjoiMjAyMy0xMi0wMSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6Ik9ybmlzIEh1bmdhcmljYSIsImlzc3VlIjoyLCJ2b2x1bWUiOjMxLCJpbXBhY3RGYWN0b3IiOiIwLjkyIn0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTMsImxhc3RQYWdlIjoyOH19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJIdW5nYXJpYW4gT3JuaXRob2xvZ2ljYWwgYW5kIE5hdHVyZSBDb25zZXJ2YXRpb24gU29jaWV0eSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MjcsImNpdGVkQnlDb3VudCI6MX19LCJxdW90ZSI6Ikhvd2V2ZXIsIGhpZ2ggbGV2ZWxzIG9mIGV1dHJvcGhpY2F0aW9uIG1heSBhbHNvIHRyaWdnZXIgdGhlIHByb2xpZmVyYXRpb24gb2YgZ3JlZW4gYWxnYWUgYW5kICBkZWNsaW5lIG9mIG1hY3JvcGh5dGVzIGxlYWRpbmcgdG8gYSBoaWdoLXR1cmJpZGl0eSBzdGF0ZSBvZiB3YXRlciwgaW4gd2hpY2ggZmV3IHNwZWNpZXMgY2FuICBzdXJ2aXZlIChTY2hlZmZlciBldCBhbC4gMTk5MykuIFRvIGFkZHJlc3MgdGhlIGV1dHJvcGhpY2F0aW9uIGFuZCBleGNlc3MgbnV0cmllbnQgaXNzdWVzIGluICBhcXVhdGljIGVjb3N5c3RlbXMsIHBoeXRvcmVtZWRpYXRpb24gb2ZmZXJzIGEgcHJvbWlzaW5nIHNvbHV0aW9uIChMaXUgZXQgYWwuIDIwMjEpLiBBcXVhdGljICBwbGFudHMgbGlrZSBkdWNrd2VlZHMsIHdhdGVyIGxldHR1Y2UgKFBpc3RpYSBzdHJhdGlvdGVzKSwgYW5kIHdhdGVyIGxpbGllcyBoYXZlIGRlbW9uc3RyYXRlZCAgdGhlaXIgYWJpbGl0eSB0byBjb21iYXQgZXV0cm9waGljYXRpb24gYnkgYWJzb3JiaW5nIGFuZCBhc3NpbWlsYXRpbmcgZXhjZXNzIHBob3NwaG9ydXMgYW5kICBuaXRyb2dlbiAoTmFoYXIgJiBIb3F1ZSAyMDIxKS4gSG93ZXZlciwgdGhlc2UgcGxhbnRzIGNhbiBzb21ldGltZXMgZ3JvdyB1bmNvbnRyb2xsYWJseSwgIHBvdGVudGlhbGx5IGNvdmVyaW5nIHRoZSB3YXRlciBzdXJmYWNlcy4gVG8gYXZvaWQgc3VjaCBzY2VuYXJpb3MsIHJlZ3VsYXIgbW9uaXRvcmluZyBpcyBlc3NlbnRpYWwgKExpdSBldCBhbC4gMjAyMSkuIFByb21vdGluZyBwdWJsaWMgYXdhcmVuZXNzIGlzIHRoZSBjb3JuZXJzdG9uZSBmb3IgbW9iaWxpemluZyBzdXBwb3J0IGFuZCBlbnN1cmluZyBlZmZlY3RpdmUgZXV0cm9waGljYXRpb24gbWFuYWdlbWVudC4gQ29udHJvbGxlZCBlcmFkaWNhdGlvbiBvZiBleGNlc3MgIGZsb3JhIHdoZW4gbmVjZXNzYXJ5IGlzIHZpdGFsIHRvIHByZXZlbnQgb3Zlcmdyb3d0aC4gVGhpcyBpbnRlZ3JhdGVkIGFwcHJvYWNoIGNhbiBlZmZlY3RpdmVseSAgcmVkdWNlIGV1dHJvcGhpY2F0aW9uIGluIHRoZSB3YXRlciBib2RpZXMgKFdhbmcgZXQgYWwuIDIwMjIpLiBFYWNoIHNwZWNpZXMgb2Ygd2V0bGFuZC1kZXBlbmRlbnQgYmlyZCBoYXMgYSB1bmlxdWUgYW5kIGNvbXBsZXggc2V0IG9mIG5lZWRzIGZvciB3ZXRsYW5kIGhhYml0YXRzIHRoYXQgbWFrZXMgaXQgZGlmZmljdWx0IHRvIGdlbmVyYWxpemUgYWJvdXQgaG93IGxvc3Mgb3IgZGVncmFkYXRpb24gb2Ygd2V0bGFuZHMgYWZmZWN0cyBiaXJkIHBvcHVsYXRpb25zLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo3OS4yOTU2MzkwMzgwODU5NCwibGVmdCI6MTAuMzAzMDA2MTcyMTgwMTc2LCJ3aWR0aCI6NzkuOTUyNzI4MjcxNDg0MzgsImhlaWdodCI6MS43MDkxOTc4Nzg4Mzc1ODU0fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjgxLjIxMjUzOTY3Mjg1MTU2LCJsZWZ0IjoxMC4zMDMwMDYxNzIxODAxNzYsIndpZHRoIjo3OS45NTkyODE5MjEzODY3MiwiaGVpZ2h0IjoxLjcwOTE5Nzg3ODgzNzU4NTR9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6ODMuMTI5NDQ3OTM3MDExNzIsImxlZnQiOjEwLjMwMzAwNjE3MjE4MDE3Niwid2lkdGgiOjc5Ljk0MjcyNjEzNTI1MzksImhlaWdodCI6MS43MDkxOTc4Nzg4Mzc1ODU0fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjg1LjA0NjM0ODU3MTc3NzM0LCJsZWZ0IjoxMC4zMDMwMDgwNzk1Mjg4MDksIndpZHRoIjo3OS45Mzk0NTMxMjUsImhlaWdodCI6MS43MDkxOTc4Nzg4Mzc1ODU0fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjg2Ljk2MzI1NjgzNTkzNzUsImxlZnQiOjEwLjMwMzAxMDk0MDU1MTc1OCwid2lkdGgiOjc5Ljk0MTg0ODc1NDg4MjgxLCJoZWlnaHQiOjEuNzA5MTk3ODc4ODM3NTg1NH19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo4OC44ODAxNjUxMDAwOTc2NiwibGVmdCI6MTAuMzAzMDEzODAxNTc0NzA3LCJ3aWR0aCI6NzkuOTM3NSwiaGVpZ2h0IjoxLjcwOTE5Nzg3ODgzNzU4NTR9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6OTAuNzk3MDY1NzM0ODYzMjgsImxlZnQiOjEwLjMwMzAxMzgwMTU3NDcwNywid2lkdGgiOjc5LjkzOTY1MTQ4OTI1NzgxLCJoZWlnaHQiOjEuNzA5MTk3ODc4ODM3NTg1NH19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo5Mi43MTM5NzM5OTkwMjM0NCwibGVmdCI6MTAuMzAzMDEzODAxNTc0NzA3LCJ3aWR0aCI6NzkuOTUwMzI1MDEyMjA3MDMsImhlaWdodCI6MS43MDkxOTc4Nzg4Mzc1ODU0fX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjEyLjIwNzk5MDY0NjM2MjMwNSwibGVmdCI6MTAuMzAzMDI0MjkxOTkyMTg4LCJ3aWR0aCI6NzkuOTUyNTE0NjQ4NDM3NSwiaGVpZ2h0Ijo3LjQ1OTkyMjc5MDUyNzM0NH19LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjoxOS44NzU2MjU2MTAzNTE1NjIsImxlZnQiOjEyLjEyMTMyNDUzOTE4NDU3LCJ3aWR0aCI6NzguMTE5MjI0NTQ4MzM5ODQsImhlaWdodCI6MS43MDkxOTc4Nzg4Mzc1ODU0fX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjIxLjc5MjUzMzg3NDUxMTcyLCJsZWZ0IjoxMC4zMDMwMjkwNjAzNjM3Nywid2lkdGgiOjc5Ljk0OTI0OTI2NzU3ODEyLCJoZWlnaHQiOjEuNzA5MTk3ODc4ODM3NTg1NH19LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjoyMy43MDk0MzY0MTY2MjU5NzcsImxlZnQiOjEwLjMwMzAyOTA2MDM2Mzc3LCJ3aWR0aCI6NzkuOTUyMjcwNTA3ODEyNSwiaGVpZ2h0IjoxLjcwOTE5Nzg3ODgzNzU4NTR9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIyNSJ9fSx7Im9yaWdpbiI6NCwid29yayI6eyJ0eXBlIjo4LCJzdWJ0eXBlIjp7Im9uZW9mS2luZCI6ImFydGljbGVUeXBlIiwiYXJ0aWNsZVR5cGUiOjF9LCJjb250cmlidXRvcnMiOlt7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBYnV6ZXIiLCJmYW1pbHkiOiLDh2VsZWtsacyHIn19XSwiaWRzIjp7Imlzc24iOltdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMjEyMDMvcnMuMy5ycy01MDc4MTc2L3YxIiwib3BlbmFsZXgiOiJXNDQwNDUyOTQxOCJ9LCJ0aXRsZSI6IlRoZSBmaXJzdCBlY28tYXNzZXNzbWVudCBvZiBwaHl0b3BsYW5rdG9uIGFzc2VtYmxhZ2VzIGFzc29jaWF0ZWQgd2l0aCBlbnZpcm9ubWVudGFsIGNvbmRpdGlvbnMgb2YgZGlmZmVyZW50IGxha2VzIGluIHRoZSBXZXN0ZXJuIEJsYWNrIFNlYSBiYXNpbiBvZiBUw7xya2l5ZSIsImFic3RyYWN0IjoiPHRpdGxlPkFic3RyYWN0PC90aXRsZT4gTGFrZXMgcGxheSBhIHZpdGFsIHJvbGUgaW4gc3VwcG9ydGluZyBiaW9kaXZlcnNpdHksIHByb3ZpZGluZyB3YXRlciByZXNvdXJjZXMsIHJlZ3VsYXRpbmcgY2xpbWF0ZSwgY3ljbGluZyBudXRyaWVudHMsIGFuZCBvZmZlcmluZyByZWNyZWF0aW9uYWwgb3Bwb3J0dW5pdGllcy4gRGVzcGl0ZSB0aGVpciBpbXBvcnRhbmNlIGZvciBlbnZpcm9ubWVudGFsIGhlYWx0aCBhbmQgaHVtYW4gd2VsbC1iZWluZywgbGFrZXMgZmFjZSBzaWduaWZpY2FudCBwcmVzc3VyZXMgaW4gdGhlIEFudGhyb3BvY2VuZSBlcmEuIFRoZSBwcmVzZW50IHdvcmsgc2Vla3MgdG8gYXNzZXNzIHRoZSBzcGVjaWVzLWVudmlyb25tZW50IGludGVyYWN0aW9ucyBhbmQgdGhlIGVjb2xvZ2ljYWwgc3RhdHVzIG9mIHNpeCBsYWtlcyBpbiB0aGUgV2VzdGVybiBCbGFjayBTZWEgYmFzaW4gb2YgVMO8cmtpeWUgdXRpbGl6aW5nIHBoeXRvcGxhbmt0b24gbWV0cmljcyBkdXJpbmcgd2V0IGFuZCBkcnkgcGVyaW9kcy4gQ2Fub25pY2FsIGNvcnJlc3BvbmRlbmNlIGFuYWx5c2lzIHJldmVhbGVkIGEgc2lnbmlmaWNhbnQgY29ycmVsYXRpb24gZXF1YWwgdG8gOTguNSUgYmV0d2VlbiBwaHl0b3BsYW5rdG9uIHNwZWNpZXMgYW5kIGVudmlyb25tZW50YWwgc3RyZXNzb3JzIGR1cmluZyB0d28gaHlkcm9sb2dpY2FsIHBlcmlvZHMuIEVsZWN0cmljYWwgY29uZHVjdGl2aXR5IChFQyksIHBILCB0b3RhbCBvcmdhbmljIGNhcmJvbiAoVE9DKSwgYW5kIHRlbXBlcmF0dXJlIHdlcmUgdGhlIG1vc3QgaW5mbHVlbnRpYWwgZW52aXJvbm1lbnRhbCBmYWN0b3JzIGFmZmVjdGluZyBwaHl0b3BsYW5rdG9uIGRpc3RyaWJ1dGlvbiBpbiBsYWtlcyAocD0wLjAwMikuIExha2UgU2FyxLFrdW0sIGEgYnJhY2tpc2ggaGFiaXRhdCwgd2FzIHVuZGVyIGhpZ2ggRUMgYW5kIHBIIHByZXNzdXJlIGFuZCBpcyBjaGFyYWN0ZXJpemVkIGJ5IHBvbGx1dGlvbi10b2xlcmFudCBzcGVjaWVzLiBMYWtlIFllbmnDp2HEn2EgYXNzb2NpYXRlZCB3aXRoIFRPQyBpcyBjaGFyYWN0ZXJpemVkIGJ5IHNvbWUgc3BlY2llcywgc3VjaCBhcyA8aXRhbGljPkFuYWJhZW5vcHNpczwvaXRhbGljPiA8aXRhbGljPm1pbGxlcmk8L2l0YWxpYz4sIDxpdGFsaWM%2BQ2hyb29jb2NjdXM8L2l0YWxpYz4gPGl0YWxpYz50dXJnaWR1czwvaXRhbGljPiw8aXRhbGljPiBQc2V1ZG9zY2hyb2VkZXJpYTwvaXRhbGljPiA8aXRhbGljPnJvYnVzdGE8L2l0YWxpYz4sIDxpdGFsaWM%2BQXBoYW5vY2Fwc2E8L2l0YWxpYz4gc3AuLCA8aXRhbGljPk1lcmlzbW9wZWRpYTwvaXRhbGljPiA8aXRhbGljPmdsYXVjYTwvaXRhbGljPiwgPGl0YWxpYz5NaWNyYWN0aW5pdW08L2l0YWxpYz4gPGl0YWxpYz5xdWFkcmlzZXR1bTwvaXRhbGljPiwgYW5kPGl0YWxpYz4gTWljcm9jeXN0aXM8L2l0YWxpYz4gPGl0YWxpYz5hZXJ1Z2lub3NhPC9pdGFsaWM%2BLiBMYWtlIEFiYW50IGlzIGxvY2F0ZWQgb24gdGhlIG9wcG9zaXRlIHNpZGUgb2YgRUMsIFRPQywgYW5kIHRlbXBlcmF0dXJlLCB3aGljaCB3YXMgYXNzb2NpYXRlZCB3aXRoIHNvbWUgc3BlY2llcyBzdWNoIGFzIDxpdGFsaWM%2BQ3ltYmVsbGEgYWZmaW5pczwvaXRhbGljPiwgPGl0YWxpYz5BY2huYW50aGlkaXVtPC9pdGFsaWM%2BIDxpdGFsaWM%2BbWludXRpc3NpbXVtPC9pdGFsaWM%2BLCA8aXRhbGljPkVuY3lvbmVtYTwvaXRhbGljPiA8aXRhbGljPm1pbnV0dW08L2l0YWxpYz4sIDxpdGFsaWM%2BRS48L2l0YWxpYz4gPGl0YWxpYz5zaWxlc2lhY3VtPC9pdGFsaWM%2BLCBhbmQ8aXRhbGljPiBEaW5vYnJ5b248L2l0YWxpYz4gPGl0YWxpYz5kaXZlcmdlbnM8L2l0YWxpYz4uIFJlc3VsdHMgb2YgdGhlIHBoeXRvLWFzc2Vzc21lbnQgZGlzcGxheWVkIHRoYXQgdGhlIGVjb2xvZ2ljYWwgc3RhdHVzIChFUykgb2YgdGhlIHNhbXBsaW5nIHN0YXRpb25zIGR1cmluZyB0aGUgcmFpbnkgYW5kIGRyeSBwZXJpb2RzIHZhcmllZCBmcm9tIGJhZCB0byBoaWdoLiBUaGUgbW9kaWZpZWQgcGh5dG9wbGFua3RvbiB0cm9waGljIGluZGV4IChNUFRJKSBleGhpYml0ZWQgdGhhdCBhIG1vZGVyYXRlIEVTIHdhcyBmb3VuZCBpbiBMYWtlcyBTYXLEsWt1bSBhbmQgWWVuacOnYcSfYSwgd2hpbGUgb3RoZXJzIGhhZCBhIGdvb2QgRVMuIFRoZSBwcmVzZW50IHN0dWR5IGNvbu%2BsgXJtZWQgdGhhdCBwaHl0b3BsYW5rdG9uIGNvbW11bml0aWVzIGFyZSBzdHJvbmdseSBsaW5rZWQgdG8gdGhlIGVjb2xvZ2ljYWwgc3RhdHVzIG9mIGxha2VzIGluIHRoZSBXZXN0ZXJuIEJsYWNrIFNlYSBiYXNpbiwgd2hpY2ggY291bGQgYmUgYXNzZXNzZWQgdXNpbmcgdGhlIE1QVEkuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjIxMjAzL3JzLjMucnMtNTA3ODE3Ni92MSIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cucmVzZWFyY2hzcXVhcmUuY29tL2FydGljbGUvcnMtNTA3ODE3Ni9sYXRlc3QucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjQtMTEtMTkiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA4IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJSZXNlYXJjaCBTcXVhcmUgKFJlc2VhcmNoIFNxdWFyZSkifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiUmVzZWFyY2ggU3F1YXJlIChVbml0ZWQgU3RhdGVzKSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NTIsImNpdGVkQnlDb3VudCI6MH19LCJxdW90ZSI6IkRlc3BpdGUgdGhlaXIgZWNvbG9naWNhbCBzaWduae%2B%2FvWNhbmNlLCB0aGVzZSB3YXRlciBib2RpZXMgYXJlIGluY3JlYXNpbmdseSB0aHJlYXRlbmVkIGJ5IGFudGhyb3BvZ2VuaWMgYWN0aXZpdGllcyAoZS5nLiwgdXJiYW5pemF0aW9uLCBhZ3JpY3VsdHVyYWwgbGFuZCB1c2UsIHdhc3Rld2F0ZXIgZGlzY2hhcmdlLCByZWVkIGN1dHRpbmcsIHNhbmQgcmVtb3ZhbCwgZXRjLikgYW5kIGNsaW1hdGUgY2hhbmdlIChIw6RkZXIgZXQgYWwuIDIwMjA7IMOHZWxla2xpIGV0IGFsLiAyMDI0KS4gTnV0cmllbnQgZW5yaWNobWVudCBpbiBsZW50aWMgZWNvc3lzdGVtcyBpbXBhY3RzIG5vdCBvbmx5IHRoZSBwaHlzaWNvY2hlbWljYWwgZmFjdG9ycyBvZiBsYWtlcyBidXQgYWxzbyBuZWdhdGl2ZWx5IGFmZmVjdHMgYXF1YXRpYyBsaWZlLCBjYXVzaW5nIGhhcm1mdWwgYWxnYWwgYmxvb21zLCBleGNlc3NpdmUgZ3Jvd3RoIG9mIG51aXNhbmNlIG1hY3JvcGh5dGVzLCBhbmQgaW5jcmVhc2VkIO%2B%2FvXNoIG1vcnRhbGl0eSwgZXNwZWNpYWxseSBzZW5zaXRpdmUgc3BlY2llcyAoU3VyZXNoIGV0IGFsLiAyMDIzKS4gSXQgYWxzbyBkaXNydXB0cyB0aGUgaHlkcm9tb3JwaG9sb2d5IGFuZCBvdmVyYWxsIGZ1bmN0aW9uaW5nIG9mIHRoZXNlIGVjb3N5c3RlbXMuIERlc3BpdGUgZ2xvYmFsIHdhdGVyIHNob3J0YWdlcywgdGhlIGdyb3dpbmcgZGVncmFkYXRpb24gb2YgZWNvc3lzdGVtcyBhbmQgdGhlIG92ZXJ1c2Ugb2YgZnJlc2h3YXRlciByZXNvdXJjZXMgaGF2ZSBzaGlmdGVkIGF0dGVudGlvbiBiYWNrIHRvIGZyZXNod2F0ZXIgZWNvc3lzdGVtcyB0byBiZXR0ZXIgbWFuYWdlIGFuZCBwcmVzZXJ2ZSB3YXRlciByZXNvdXJjZXMgZ2xvYmFsbHkgKMOHZWxla2xpIGV0IGFsLiAyMDIxOyBTdXJlc2ggZXQgYWwuIDIwMjMpLiBBc3Nlc3NpbmcgdGhlIGVjb2xvZ2ljYWwgc3RhdHVzIG9mIGxha2VzIHRocm91Z2ggcGh5dG9wbGFua3RvbiBtZXRyaWNzIGlzIGEgY3J1Y2lhbCBjb21wb25lbnQgb2YgZW52aXJvbm1lbnRhbCBtb25pdG9yaW5nIGFuZCBtYW5hZ2VtZW50LCBhcyBpdCBhbGlnbnMgd2l0aCB0aGUgZ3VpZGVsaW5lcyBzZXQgYnkgdGhlIFdhdGVyIEZyYW1ld29yayBEaXJlY3RpdmUgKERpcmVjdGl2ZSAyMDAwKS4gUGh5dG9wbGFua3RvbiwgYXMgcHJpbWFyeSBwcm9kdWNlcnMsIGFyZSBzZW5zaXRpdmUgaW5kaWNhdG9ycyBvZiBjaGFuZ2VzIGluIHdhdGVyIHF1YWxpdHkgYW5kIGVjb3N5c3RlbSBoZWFsdGggKFJleW5vbGRzIGV0IGFsLiAyMDAyOyDDh2VsZWtsaSBldCBhbC4gMjAyM2EpLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjY2Ljk2NDIyNTc2OTA0Mjk3LCJsZWZ0Ijo0LjUzNDMxMzY3ODc0MTQ1NSwid2lkdGgiOjkwLjYxNDc2MTM1MjUzOTA2LCJoZWlnaHQiOjEuNzc1NTI5NjIzMDMxNjE2Mn19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjY5LjIzNjkwNzk1ODk4NDM4LCJsZWZ0Ijo0LjUzNDMxMzY3ODc0MTQ1NSwid2lkdGgiOjkwLjAxMDMwNzMxMjAxMTcyLCJoZWlnaHQiOjEuNzc1NTI5NjIzMDMxNjE2Mn19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjcxLjUwOTU5MDE0ODkyNTc4LCJsZWZ0Ijo0LjUzNDMxMzY3ODc0MTQ1NSwid2lkdGgiOjg1LjgxMjE3OTU2NTQyOTY5LCJoZWlnaHQiOjEuNzc1NTI5NjIzMDMxNjE2Mn19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjczLjc4MjI3MjMzODg2NzE5LCJsZWZ0Ijo0LjUzNDMxMzY3ODc0MTQ1NSwid2lkdGgiOjg2LjA3MDMxMjUsImhlaWdodCI6MS43NzU1Mjk2MjMwMzE2MTYyfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NzYuMDU0OTU0NTI4ODA4NiwibGVmdCI6NC41MzQzMTM2Nzg3NDE0NTUsIndpZHRoIjo4OS42MjM2NjQ4NTU5NTcwMywiaGVpZ2h0IjoxLjc3NTUyOTYyMzAzMTYxNjJ9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo3OC4zMjc2MjkwODkzNTU0NywibGVmdCI6NC41MzQzMTM2Nzg3NDE0NTUsIndpZHRoIjo4OS44NTk3NTY0Njk3MjY1NiwiaGVpZ2h0IjoxLjc3NTUyOTYyMzAzMTYxNjJ9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo4MC42MDAzMTg5MDg2OTE0LCJsZWZ0Ijo0LjUzNDMxMzY3ODc0MTQ1NSwid2lkdGgiOjg5LjY0MDgzODYyMzA0Njg4LCJoZWlnaHQiOjEuNzc1NTI5NjIzMDMxNjE2Mn19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjgyLjg3Mjk5MzQ2OTIzODI4LCJsZWZ0Ijo0LjUzNDMxMzY3ODc0MTQ1NSwid2lkdGgiOjkwLjgzNjYzOTQwNDI5Njg4LCJoZWlnaHQiOjQuMDQ4MTk0ODg1MjUzOTA2fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6ODcuNDE4MzQyNTkwMzMyMDMsImxlZnQiOjQuNTM0MzEzNjc4NzQxNDU1LCJ3aWR0aCI6MzMuNzkwOTM1NTE2MzU3NDIsImhlaWdodCI6MS43NzU1Mjk2MjMwMzE2MTYyfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6OTEuMjA2MTQ2MjQwMjM0MzgsImxlZnQiOjQuNTM0MzEzNjc4NzQxNDU1LCJ3aWR0aCI6ODQuNjc1Mzc2ODkyMDg5ODQsImhlaWdodCI6MS43NzU1Mjk2MjMwMzE2MTYyfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6OTMuNDc4ODIwODAwNzgxMjUsImxlZnQiOjQuNTM0MzEzNjc4NzQxNDU1LCJ3aWR0aCI6ODkuOTgzODQ4NTcxNzc3MzQsImhlaWdodCI6MS43NzU1Mjk2MjMwMzE2MTYyfX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6NC43NDk2NDM4MDI2NDI4MjIsImxlZnQiOjQuNTM0MzEzNjc4NzQxNDU1LCJ3aWR0aCI6ODcuOTU3ODYyODU0MDAzOSwiaGVpZ2h0IjoxLjc3NTUzMDU3NjcwNTkzMjZ9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo3LjAyMjMwOTMwMzI4MzY5MSwibGVmdCI6NC41MzQzMTM2Nzg3NDE0NTUsIndpZHRoIjo5MC42NjY1MTE1MzU2NDQ1MywiaGVpZ2h0IjoxLjc3NTUzMDU3NjcwNTkzMjZ9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIyIn19XQ%3D%3D.

These processes underscore eutrophication's self-reinforcing nature, demanding targeted interventions like P-precipitation or biomanipulation. Predictive models warn 80% of Indian lakes could become unusable by 2030 without curbing nutrient fluxes (Nair & Nayak, 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hdnlhIFZpa3JhbWFuIiwiZmFtaWx5IjoiTmFpciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQcmF0ZWVwIEt1bWFyIiwiZmFtaWx5IjoiTmF5YWsifX1dLCJpZHMiOnsiaXNzbiI6WyIyMjk2LTc3NDUiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzODkvZm1hcnMuMjAyMy4xMDg3Mjk2Iiwib3BlbmFsZXgiOiJXNDM4MjcyMzEzMyJ9LCJ0aXRsZSI6IlVuY292ZXJpbmcgd2F0ZXIgcXVhbGl0eSBhbmQgZXZhbHVhdGluZyB2dWxuZXJhYmlsaXRpZXMgb2Ygc21hbGwtc2NhbGUgZmlzaGVyaWVzIGluIENoaWxpa2EgTGFnb29uLCBJbmRpYSIsImFic3RyYWN0IjoiU21hbGwtc2NhbGUgZmlzaGVyaWVzIChTU0ZzKSBwbGF5IGEgc3Ryb25nIHJvbGUgaW4gc3VzdGFpbmluZyBtaWxsaW9ucyBvZiBsaXZlbGlob29kcywgZm9vZCBzZWN1cml0eSwgbnV0cml0aW9uLCBhbmQgaW5jb21lIGdsb2JhbGx5IGJ1dCB0aGUgZmlzaGVycyBlbmdhZ2VkIGluIHRoaXMgc2VjdG9yIHNpbXVsdGFuZW91c2x5IGV4cGVyaWVuY2UgaGlnaCBsZXZlbHMgb2YgdnVsbmVyYWJpbGl0eSBhbmQgcHJvY2Vzc2VzIG9mIG1hcmdpbmFsaXNhdGlvbi4gU2V2ZXJhbCBmYWN0b3JzIGFyZSBhdHRyaWJ1dGVkIHRvIHRoZSBtdWx0aWRpbWVuc2lvbmFsIHZ1bG5lcmFiaWxpdGllcyB0aGUgc21hbGwtc2NhbGUgZmlzaGVycyBleHBlcmllbmNlLCBzcGFubmluZyBib3RoIG5hdHVyYWwgKGUuZy4sIG5hdHVyYWwgZGlzYXN0ZXJzLCBlY29zeXN0ZW0gY2hhbmdlKSBhbmQgYW50aHJvcG9nZW5pYyAoZS5nLiwgcG9saWN5IGNoYW5nZSwgaHlkcm9sb2dpY2FsIGludGVydmVudGlvbnMsIGFxdWFjdWx0dXJlKSBwcmVzc3VyZXMuIFdoaWxlIHRoZXJlIGlzIG11Y2ggbGl0ZXJhdHVyZSBvbiB2YXJpb3VzIG5hdHVyYWwgYW5kIGh1bWFuIGRyaXZlcnMgb2YgdnVsbmVyYWJpbGl0eSBpbiBzbWFsbC1zY2FsZSBmaXNoZXJ5IGNvbW11bml0aWVzLCBhbiBhYnNlbmNlIG9mIHJlc2VhcmNoIGNvbm5lY3RpbmcgdnVsbmVyYWJpbGl0eSB3aXRoIHdhdGVyIHF1YWxpdHkgaXMgZXZpZGVudC4gRmlzaGVyIGNvbW11bml0aWVzIG9mdGVuIHRhbGsgYWJvdXQgZmlzaCBpbiByZWxhdGlvbiB0byB0aGUgaGVhbHRoIG9mIHRoZWlyIGFxdWF0aWMgaGFiaXRhdHMgd2hlcmVpbiB3YXRlciBxdWFsaXR5IGlzIHNlZW4gYXMgYSBrZXkgcGFyYW1ldGVyLiBUaGUgbGluayBiZXR3ZWVuIGhlYWx0aHkgZmlzaCBhbmQgZ29vZCBxdWFsaXR5IHdhdGVyIGhhcyBzaWduaWZpY2FudCBpbXBsaWNhdGlvbnMgZm9yIHN0cm9uZyBhbmQgdmlhYmxlIGZpc2hpbmcgY29tbXVuaXRpZXMuIFRoaXMgcGFwZXIgZXhhbWluZXMgdGhlc2UgbGlua3MgZnVydGhlciBieSBmb2N1c2luZyBvbiB0aGUgbmF0dXJlIG9mIHZ1bG5lcmFiaWxpdGllcyBjYXVzZWQgYnkgd2F0ZXIgcXVhbGl0eSBjaGFuZ2VzIGluIHRoZSBzbWFsbC1zY2FsZSBmaXNoZXJ5IHN5c3RlbSBvZiBDaGlsaWthIExhZ29vbiBpbiBJbmRpYS4gV2UgdW5kZXJ0YWtlIGRldGFpbGVkIGFuYWx5c2lzIG9mIHRoZSBpbnZhc2l2ZSBzaHJpbXAgYXF1YWN1bHR1cmUgYWN0aXZpdGllcyBhbmQgaHlkcm9sb2dpY2FsIGludGVydmVudGlvbnMgZm9yIG9wZW5pbmcgb2YgYSBsYWdvb24gaW5sZXQgd2l0aCB0aGUgQmF5IG9mIEJlbmdhbCBhcyB0d28gZG9taW5hbnQgZHJpdmVycyBhZHZlcnNlbHkgaW1wYWN0aW5nIHdhdGVyIHF1YWxpdHkgYW5kIGluY3JlYXNpbmcgdnVsbmVyYWJpbGl0aWVzIG9mIHRoZSBlbnRpcmUgc21hbGwtc2NhbGUgZmlzaGVyaWVzIHNvY2lhbC1lY29sb2dpY2FsIHN5c3RlbS4gT3VyIGFuYWx5c2lzIHN1Z2dlc3RzIHRoYXQgdGhlcmUgYXJlIHN0cm9uZyBpbnRlcmNvbm5lY3Rpb25zIGJldHdlZW4gY2hhbmdlcyBpbiB3YXRlciBxdWFsaXR5IGFuZCB0aGUgbGV2ZWxzIG9mIHZ1bG5lcmFiaWxpdGllcyBpbiB0aGUgU1NGcyBvZiBDaGlsaWthIExhZ29vbi4gUG9sbHV0YW50cyBzdWNoIGFzIHBlc3RpY2lkZXMsIGFuZCBvcmdhbmljIGNvbXBvdW5kcyBhY2N1bXVsYXRlIGluIGZpc2ggdGlzc3VlcyBhbmQgYWZmZWN0IHRoZWlyIGdyb3d0aCwgcmVwcm9kdWN0aW9uLCBhbmQgb3ZlcmFsbCBoZWFsdGguIFRoaXMgbGVkIHRvIGRlY2xpbmVzIGluIGZpc2ggcG9wdWxhdGlvbnMsIG1ha2luZyBpdCBtb3JlIGRpZmZpY3VsdCBmb3IgZmlzaGVycyB0byBtYWtlIGEgbGl2aW5nLiBJbiBhZGRpdGlvbiB0byBkaXJlY3QgaW1wYWN0cyBvbiBmaXNoIHBvcHVsYXRpb25zLCBwb29yIHdhdGVyIHF1YWxpdHkgYWxzbyBoYXMgaW5kaXJlY3QgZWZmZWN0cyBvbiB0aGUgc29jaWFsIGFuZCBlY29ub21pYyB2dWxuZXJhYmlsaXR5IG9mIFNTRnMuIEZvciBleGFtcGxlLCBjb250YW1pbmF0aW9uIG9mIHdhdGVyIHNvdXJjZXMgbGVkIHRvIHRoZSByZWR1Y2VkIG51bWJlciBvZiBmaXNoIHNwZWNpZXMgcmVkdWNpbmcgdGhlIGFtb3VudCBvZiB0aW1lIGZpc2hlcnMgY2FuIHNwZW5kIG9uIGZpc2hpbmcgYWN0aXZpdGllcy4gVGhpcyBhbHNvIGFmZmVjdGVkIHRoZSBtYXJrZXRhYmlsaXR5IG9mIGZpc2ggcHJvZHVjdHMsIHJlZHVjZWQgaW5jb21lIGFuZCBpbmNyZWFzZWQgcG92ZXJ0eS4gVG8gZnVsbHkgdW5kZXJzdGFuZCB0aGUgaW50ZXJjb25uZWN0aW9ucyBiZXR3ZWVuIHdhdGVyIHF1YWxpdHkgYW5kIHZ1bG5lcmFiaWxpdHkgaW4gU1NGcyBpbiBDaGlsaWthIExhZ29vbiwgaXQgaXMgaW1wb3J0YW50IHRvIGNvbnNpZGVyIGJvdGggZW52aXJvbm1lbnRhbCBhbmQgc29jaWFsIGZhY3RvcnMsIGFzIHdlbGwgYXMgdGhlIGNvbXBsZXggZmVlZGJhY2sgbG9vcHMgYmV0d2VlbiB0aGVzZSBmYWN0b3JzLiBUaGUgc3R1ZHkgaGVscHMgaW4gYnJpZGdpbmcgYSBjcnVjaWFsIGdhcCBpbiBvdXIgdW5kZXJzdGFuZGluZyBvZiB0aGUgcm9sZSBvZiB3YXRlciBxdWFsaXR5IGluIHZ1bG5lcmFiaWxpdHkgYW5hbHlzaXMgd2l0aGluIHJlc291cmNlIGRlcGVuZGVudCBjb21tdW5pdGllcy4gV2UgY29uY2x1ZGUgd2l0aCBrZXkgaW5zaWdodHMgb24gcG9zc2libGUgY29waW5nIHJlc3BvbnNlcyBhbmQgYWRhcHRpdmUgY2FwYWNpdHkgbmVjZXNzYXJ5IGZvciB0aGUgc21hbGwtc2NhbGUgZmlzaGVyaWVzIGNvbW11bml0aWVzIHRvIHRyYW5zaXRpb24gdG93YXJkIHZpYWJpbGl0eS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMzM4OS9mbWFycy4yMDIzLjEwODcyOTYiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vd3d3LmZyb250aWVyc2luLm9yZy9hcnRpY2xlcy8xMC4zMzg5L2ZtYXJzLjIwMjMuMTA4NzI5Ni9wZGYiLCJpc3N1ZWREYXRlIjoiMjAyMy0wNi0yOSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZyb250aWVycyBpbiBNYXJpbmUgU2NpZW5jZSIsInZvbHVtZSI6MTAsImltcGFjdEZhY3RvciI6IjQuMjgifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRnJvbnRpZXJzIE1lZGlhIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo1MSwiY2l0ZWRCeUNvdW50Ijo1fX0sInF1b3RlIjoiV2F0ZXIgcXVhbGl0eSBvZiBDaGlsaWthIExhZ29vbiBoYXMgZGVncmFkZWQgZHVlIHRvIGFudGhyb3BvZ2VuaWMgaW50ZXJ2ZW50aW9ucyBhbmQgbmF0dXJhbCBmb3JjZXMgb2YgY2hhbmdlLiBWYXJpb3VzIGFjdGl2aXRpZXMgaW4gdGhlIENoaWxpa2EgTGFnb29uLCBzdWNoIGFzIGluZHVzdHJpYWwgd2FzdGV3YXRlciBkaXNwb3NhbCwgc2V3YWdlIGR1bXBpbmcsIGFxdWFjdWx0dXJlLCBoeWRyb2xvZ2ljYWwgaW50ZXJ2ZW50aW9ucywgYW5kIGN5Y2xvbmVzLCBhcmUgcHJvZHVjaW5nIHNhbGluaXR5IGNoYW5nZXMsIHNlZGltZW50IGRlcG9zaXRpb24sIG51dHJpZW50IGVucmljaG1lbnQsIGV1dHJvcGhpY2F0aW9uLCBhbmQgZGVhZCB6b25lcywgYXMgc3RhdGVkLiBGaXNoIHN1cnZpdmFsIGlzIGF0IHJpc2sgYXMgYSByZXN1bHQgb2YgdGhlIHNpZ25pZmljYW50IHJlZHVjdGlvbiBpbiBlbnZpcm9ubWVudGFsIGNpcmN1bXN0YW5jZXMsIHdoaWNoIGRpbWluaXNoZXMgZmlzaGVybWVu4oCZcyBpbmNvbWUgYW5kIHB1c2hlcyB0aGVtIG91dCBvZiBzb2NpZXR5LiBUaGVzZSBjYXVzZXMgYWZmZWN0aW5nIHRoZSB3YXRlciBxdWFsaXR5IG9mIHRoZSBMYWdvb24gZGVwaWN0IG1hbnkgZmFjZXMgb2YgdnVsbmVyYWJpbGl0eSBpbiBTU0YgY29tbXVuaXRpZXMuIEFzIHRoZSBDaGlsaWthIExhZ29vbiBlbnZpcm9ubWVudCBpcyBleHBlY3RlZCB0byB1bmRlcmdvIGluY3JlYXNpbmdseSBkcmFzdGljIGNoYW5nZXMgKHN1Y2ggYXMgY2F0Y2ggY29tcG9zaXRpb24sIGNhdGNoIGNhcGFjaXR5LCBiaW9kaXZlcnNpdHkgdmFyaWF0aW9ucywgd2F0ZXIgcXVhbGl0eSwgYW5kIGZpc2hlcnkgcmV2ZW51ZXMpLCBpbnRlcm5hdGlvbmFsIHBvbGljeSBoYXMgYmVjb21lIG1vcmUgaW50ZXJlc3RlZCBpbiBhZGFwdGF0aW9uIG1lYXN1cmVzLiBHaXZlbiBob3cgZGl2ZXJzZSBTU0YgZWNvc3lzdGVtcyBhcmUgZXhwZWN0ZWQgdG8gYmUgYmV0d2VlbiBjb250aW5lbnRzLCB0aGVyZSBpcyBubyDigJxvbmUgc2l6ZSBmaXRzIGFsbOKAnSBhcHByb2FjaC4gVGhlIHBhcGVyIG91dGxpbmVkIHNldmVyYWwgY29waW5nIGFuZCBhZGFwdGF0aW9uIG1lY2hhbmlzbXMgdG8gaW1wcm92ZSB3YXRlciBxdWFsaXR5IGFuZCBjb250aW51ZSBmaXNoaW5nIGluIGEgc3VzdGFpbmFibGUgd2F5LiBBZGFwdGF0aW9uIHN0cmF0ZWdpZXMgY2FuIG9jY2FzaW9uYWxseSBoYXZlIHVuZm9yZXNlZW4gY29uc2VxdWVuY2VzIGZvciBmaXNoZXJpZXMgYW5kIGZpc2hpbmcgY29tbXVuaXRpZXMuIEFueSB1bmV4cGVjdGVkIGNvbnNlcXVlbmNlcyBvZiBhZGFwdGF0aW9uIGNhbiBiZSBtaXRpZ2F0ZWQgdGhyb3VnaCBsb25nLXRlcm0gcGxhbm5pbmcgYW5kIGlkZW50aWZpY2F0aW9uIG9mIGZ1dHVyZSBvcHRpb25zIHRocm91Z2ggc2NlbmFyaW8gYW5hbHlzaXMuIFRoZSBmaW5kaW5ncyBvZiB0aGlzIHBhcGVyIGNhbiBiZSB1dGlsaXplZCB0byBkZXZlbG9wIHBvbGljaWVzIHRvIHByb3RlY3Qgd2F0ZXIgcXVhbGl0eSBhbmQgU1NGcyBpbiBhIHZhcmlldHkgb2YgY29hc3RhbCBsYWdvb24gc2V0dGluZ3MuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjIyLjI4NTAzNjA4NzAzNjEzMywibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ5MzA1MzQzNjI3OTMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjIzLjc3MzUxOTUxNTk5MTIxLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDgzMzY0MTA1MjI0NjEsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjI1LjI1NTU4NDcxNjc5Njg3NSwibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ4OTU1OTE3MzU4Mzk4NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjYuNzQ0MDY4MTQ1NzUxOTUzLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNTA2ODA1NDE5OTIxODc1LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyOC4yMjYxMzMzNDY1NTc2MTcsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40OTA0NjMyNTY4MzU5NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjkuNzE0NjE2Nzc1NTEyNjk1LCJsZWZ0Ijo4LjMzMTU5MjU1OTgxNDQ1Mywid2lkdGgiOjQwLjQ5NDA5ODY2MzMzMDA4LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozMS4xOTY2ODM4ODM2NjY5OTIsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40OTQzOTIzOTUwMTk1MywiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MzIuNjg1MTY1NDA1MjczNDQsImxlZnQiOjguMzMxNTk2Mzc0NTExNzE5LCJ3aWR0aCI6NDAuNDg3NDQ1ODMxMjk4ODMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM0LjE2NzIzMjUxMzQyNzczNCwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40OTc3NDU1MTM5MTYwMTYsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM1LjY1NTcxNTk0MjM4MjgxLCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ5ODYzODE1MzA3NjE3LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozNy4xMzc3ODMwNTA1MzcxMSwibGVmdCI6MTAuNjg1MTg2Mzg2MTA4Mzk4LCJ3aWR0aCI6MzguMTQxMTA1NjUxODU1NDcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM4LjYyNjI2NjQ3OTQ5MjE5LCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4NzE1MjA5OTYwOTM3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDAuMTA4MzMzNTg3NjQ2NDg0LCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4OTg1NjcxOTk3MDcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQxLjU5NjgxMzIwMTkwNDMsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDg4NjY2NTM0NDIzODMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQzLjA3ODg4MDMxMDA1ODU5NCwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40ODE4NjExMTQ1MDE5NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDQuNTY3MzYzNzM5MDEzNjcsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDgzMDU4OTI5NDQzMzYsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQ2LjA0OTQzMDg0NzE2Nzk3LCJsZWZ0Ijo4LjMzMzEwODkwMTk3NzUzOSwid2lkdGgiOjQwLjUwMDc0NzY4MDY2NDA2LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo0Ny41Mzc5MTQyNzYxMjMwNSwibGVmdCI6OC4zMzMxMDg5MDE5Nzc1MzksIndpZHRoIjo0MC40OTU2MDU0Njg3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDkuMDE5OTgxMzg0Mjc3MzQ0LCJsZWZ0Ijo4LjMzMzEwNjA0MDk1NDU5LCJ3aWR0aCI6NDAuNTA2ODE2ODY0MDEzNjcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUwLjUwODQ2NDgxMzIzMjQyLCJsZWZ0Ijo4LjMzMzEwNjA0MDk1NDU5LCJ3aWR0aCI6NDAuNDg4NTA2MzE3MTM4NjcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUxLjk5MDUyODEwNjY4OTQ1LCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDk3NzI2NDQwNDI5NjksImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUzLjQ3OTAxMTUzNTY0NDUzLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6MzMuMTU5NDQ2NzE2MzA4NTk0LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo1NC45NjEwNzg2NDM3OTg4MywibGVmdCI6MTAuNjgzNjcwMDQzOTQ1MzEyLCJ3aWR0aCI6MzguMTM2NTk2Njc5Njg3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NTYuNDQ5NTYyMDcyNzUzOTA2LCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDg3MTQ0NDcwMjE0ODQ0LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEzIn19XQ%3D%3D. Understanding this interplay chemical loading sparking biological overgrowth and feedback loops is crucial for resilient management, bridging to sector-specific drivers like agriculture and sewage. The eutrophication process linking nutrient enrichment, bloom formation, and oxygen depletion is illustrated in Figure 2.

Figure 2: The Eutrophication Process

Intensified agricultural practices and urbanization frequently alter the stoichiometric balance of these inputs, often causing phosphorus loading to exceed nitrogen levels and thereby driving significant shifts in water chemistry and ecosystem dynamics (Haq et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjQsInN1YnR5cGUiOnt9LCJjb250cmlidXRvcnMiOlt7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJadWxmcWFydWwiLCJmYW1pbHkiOiJIYXEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU25vYmVyIiwiZmFtaWx5IjoiSXJzaGFkIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkF6bWF0IEFsYW0iLCJmYW1pbHkiOiJLaGFuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNoZWlraCBGLiIsImZhbWlseSI6IkFobWFkIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNob3drZWVuIE11emFtaWwiLCJmYW1pbHkiOiJCYXNoaXIifX1dLCJpZHMiOnsiaXNzbiI6WyIyNjMyLTA1MTciXSwiaXNibiI6W10sImRvaSI6IjEwLjU3NzIvaW50ZWNob3Blbi4xMDA2NTc4Iiwib3BlbmFsZXgiOiJXNDQwNDUyOTIwMyIsIm9wZW5BY2Nlc3NJZCI6IjEwMjk5MDMyMjI1MDgzODQ2ODU2In0sInRpdGxlIjoiQWR2YW5jZXMgaW4gTWFuYWdpbmcgTml0cm9nZW4gYW5kIFBob3NwaG9ydXMgRW1pc3Npb25zIGluIFJ1bWluYW50czogQSBIb2xpc3RpYyBBcHByb2FjaCIsImFic3RyYWN0Ijoi4oCcQmFsYW5jaW5nIE51dHJpdGlvbiBhbmQgRW52aXJvbm1lbnRhbCBTdXN0YWluYWJpbGl0eTogQWR2YW5jZXMgaW4gTWFuYWdpbmcgTml0cm9nZW4gYW5kIFBob3NwaG9ydXMgRW1pc3Npb25zIGluIFJ1bWluYW50c%2BKAnSBleHBsb3JlcyB0aGUgaW50cmljYXRlIHJlbGF0aW9uc2hpcCBiZXR3ZWVuIHJ1bWluYW50IG51dHJpdGlvbiBhbmQgZW52aXJvbm1lbnRhbCBzdGV3YXJkc2hpcC4gSXQgZGVsdmVzIGludG8gdGhlIGR5bmFtaWNzIG9mIG5pdHJvZ2VuIGFuZCBwaG9zcGhvcnVzIG1ldGFib2xpc20gaW4gcnVtaW5hbnRzLCBhZGRyZXNzaW5nIHRoZSBlbnZpcm9ubWVudGFsIHJlcGVyY3Vzc2lvbnMgb2YgdGhlaXIgZW1pc3Npb25zLiBUaGVzZSBlbWlzc2lvbnMsIHByaW1hcmlseSBpbiB0aGUgZm9ybSBvZiBhbW1vbmlhIGFuZCBwaG9zcGhhdGVzLCBwb3NlIHNpZ25pZmljYW50IGVudmlyb25tZW50YWwgY2hhbGxlbmdlcywgY29udHJpYnV0aW5nIHRvIGFpciBhbmQgd2F0ZXIgcG9sbHV0aW9uLCBzb2lsIGRlZ3JhZGF0aW9uLCBhbmQgY2xpbWF0ZSBjaGFuZ2UuIFRoZSBjaGFwdGVyIGhpZ2hsaWdodHMgaW5ub3ZhdGl2ZSBzdHJhdGVnaWVzIGFuZCB0ZWNobm9sb2dpY2FsIGJyZWFrdGhyb3VnaHMgYWltZWQgYXQgbWl0aWdhdGluZyBuaXRyb2dlbiBhbmQgcGhvc3Bob3J1cyBleGNyZXRpb24sIGNydWNpYWwgZm9yIHJlZHVjaW5nIGVudmlyb25tZW50YWwgcG9sbHV0aW9uLiBJdCBvZmZlcnMgaW5zaWdodHMgaW50byBkaWV0YXJ5IGFkanVzdG1lbnRzLCBmZWVkIGFkZGl0aXZlcywgYW5kIGJlc3QgcHJhY3RpY2VzIGZvciBzdXN0YWluYWJsZSBydW1pbmFudCBwcm9kdWN0aW9uLiBLZXkgYXJlYXMgb2YgZm9jdXMgaW5jbHVkZSBkaWV0YXJ5IGFkanVzdG1lbnRzIHRhaWxvcmVkIHRvIG9wdGltaXplIG51dHJpZW50IHV0aWxpemF0aW9uIGFuZCByZWR1Y2Ugd2FzdGUsIHRoZSB1dGlsaXphdGlvbiBvZiBmZWVkIGFkZGl0aXZlcyB0byBlbmhhbmNlIG51dHJpZW50IGFic29ycHRpb24gYW5kIG1pbmltaXplIGV4Y3JldGlvbiwgYW5kIHRoZSBpbXBsZW1lbnRhdGlvbiBvZiBiZXN0IG1hbmFnZW1lbnQgcHJhY3RpY2VzIHRvIHByb21vdGUgc3VzdGFpbmFibGUgcnVtaW5hbnQgcHJvZHVjdGlvbi4gQnkgaW50ZWdyYXRpbmcgc2NpZW50aWZpYyBmaW5kaW5ncyB3aXRoIHByYWN0aWNhbCBhcHBsaWNhdGlvbnMsIHRoaXMgcmV2aWV3IHVuZGVyc2NvcmVzIHRoZSBpbXBvcnRhbmNlIG9mIGFjaGlldmluZyBoYXJtb255IGJldHdlZW4gbnV0cml0aW9uIG9wdGltaXphdGlvbiBhbmQgZW52aXJvbm1lbnRhbCByZXNwb25zaWJpbGl0eSBpbiBtb2Rlcm4gYWdyaWN1bHR1cmUuIFRocm91Z2ggdGhpcyBjb21wcmVoZW5zaXZlIGFuYWx5c2lzLCB0aGUgY2hhcHRlciBhaW1zIGZvciBiYWxhbmNpbmcgdGhlIGR1YWwgZ29hbHMgb2Ygb3B0aW1pemluZyBydW1pbmFudCBudXRyaXRpb24gYW5kIG1pbmltaXppbmcgZW52aXJvbm1lbnRhbCBpbXBhY3QsIHRoZXJlYnkgY29udHJpYnV0aW5nIHRvIG1vcmUgc3VzdGFpbmFibGUgYW5kIHJlc2lsaWVudCBhZ3JpY3VsdHVyYWwgc3lzdGVtcy4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuNTc3Mi9pbnRlY2hvcGVuLjEwMDY1NzgiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vd3d3LmludGVjaG9wZW4uY29tL2NpdGF0aW9uLXBkZi11cmwvMTE4NzgxNCIsImlzc3VlZERhdGUiOiIyMDI0LTExLTE1IiwiYWNjZXNzZWREYXRlIjoiMjAyNS0wOSIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiVmV0ZXJpbmFyeSBtZWRpY2luZSBhbmQgc2NpZW5jZSIsImltcGFjdEZhY3RvciI6IjAuOTkifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiSW50ZWNoT3BlbiJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NTIsImNpdGVkQnlDb3VudCI6MX19LCJxdW90ZSI6IldhdGVyIFF1YWxpdHkgQ2hhbmdlcyDigKIgQ2xhcml0eSBhbmQgQWxnYWwgQmxvb21zOiBXYXRlciBjbGFyaXR5IG1heSBkZWNyZWFzZSBzbGlnaHRseSBkdWUgdG8gdGhlIGluaXRpYWwgYWxnYWwgYmxvb21zLiBIb3dldmVyLCB0aGlzIHBoYXNlIGlzIG9mdGVuIHN0aWxsIGNoYXJhY3Rlcml6ZWQgYnkgcmVsYXRpdmVseSBnb29kIHdhdGVyIHF1YWxpdHkuIOKAoiBCaW9kaXZlcnNpdHk6IFRoZSBlY29zeXN0ZW0gbWlnaHQgc3RpbGwgc3VwcG9ydCBhIGRpdmVyc2UgcmFuZ2Ugb2YgYXF1YXRpYyBvcmdhbmlzbXMsIGluY2x1ZGluZyBmaXNoIGFuZCBpbnZlcnRlYnJhdGVzLiAzLjEuNC4yIFBoYXNlIDI6IE1lc290cm9waGljIHRvIGV1dHJvcGhpYyB0cmFuc2l0aW9uIDEuIE51dHJpZW50IEltYmFsYW5jZSDigKIgUGhvc3Bob3J1cyB2cy4gTml0cm9nZW46IEFzIGFncmljdWx0dXJhbCBwcmFjdGljZXMgaW50ZW5zaWZ5IGFuZCB1cmJhbml6YXRpb24gaW5jcmVhc2VzLCBwaG9zcGhvcnVzIGlucHV0cyAoZS5nLiwgZnJvbSBmZXJ0aWxpemVycyBhbmQgZGV0ZXJnZW50cykgb2Z0ZW4gZXhjZWVkIHRob3NlIG9mIG5pdHJvZ2VuLiBUaGlzIGltYmFsYW5jZSBjYW4gZHJpdmUgc2lnbmlmaWNhbnQgY2hhbmdlcyBpbiB3YXRlciBjaGVtaXN0cnkgYW5kIGVjb3N5c3RlbSBkeW5hbWljcy4gMi4gQWxnYWwgQmxvb20gSW50ZW5zaWZpY2F0aW9uIOKAoiBDeWFub2JhY3RlcmlhIERvbWluYW5jZTogRXhjZXNzIHBob3NwaG9ydXMgY2FuIGxlYWQgdG8gdGhlIGRvbWluYW5jZSBvZiBjeWFub2JhY3RlcmlhIChibHVlLWdyZWVuIGFsZ2FlKSwgd2hpY2ggY2FuIG91dGNvbXBldGUgb3RoZXIgYWxnYWUgYW5kIGFxdWF0aWMgcGxhbnRzLiBUaGVzZSBibG9vbXMgY2FuIGJlIHRveGljIGFuZCBoYXJtZnVsIHRvIGFxdWF0aWMgbGlmZS4g4oCiIEh5cG94aWE6IFRoZSBkZWNvbXBvc2l0aW9uIG9mIGxhcmdlIGFsZ2FsIGJsb29tcyBjb25zdW1lcyBveHlnZW4sIGxlYWRpbmcgdG8gaHlwb3hpYyAobG93IG94eWdlbikgb3IgYW5veGljIChubyBveHlnZW4pIGNvbmRpdGlvbnMsIHBhcnRpY3VsYXJseSBpbiBkZWVwZXIgd2F0ZXJzLiAzLiBXYXRlciBRdWFsaXR5IERldGVyaW9yYXRpb24g4oCiIERlY3JlYXNlZCBDbGFyaXR5OiBXYXRlciBjbGFyaXR5IHNpZ25pZmljYW50bHkgZGVjcmVhc2VzIGR1ZSB0byBkZW5zZSBhbGdhbCBncm93dGguICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjY3LjI2MTAwMTU4NjkxNDA2LCJsZWZ0IjoxNS41ODM0NTg5MDA0NTE2Niwid2lkdGgiOjIwLjQ2MDYxODk3Mjc3ODMyLCJoZWlnaHQiOjEuOTA1MTQ4NjI1MzczODQwM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo3MC41MTMwMzEwMDU4NTkzOCwibGVmdCI6MTcuMzk4MjE2MjQ3NTU4NTk0LCJ3aWR0aCI6NjYuNDAzNTQ5MTk0MzM1OTQsImhlaWdodCI6MS45MDUxNTcwODkyMzMzOTg0fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjcyLjEzOTA0NTcxNTMzMjAzLCJsZWZ0IjoxOS4wODgzNTYwMTgwNjY0MDYsIndpZHRoIjo2NS43ODg1MjA4MTI5ODgyOCwiaGVpZ2h0IjozLjUyODQ2MDUwMjYyNDUxMTd9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NzcuMDE3MDk3NDczMTQ0NTMsImxlZnQiOjE3LjM5ODIxNjI0NzU1ODU5NCwid2lkdGgiOjYyLjA5NDU3MDE1OTkxMjExLCJoZWlnaHQiOjEuOTA1MTU3MDg5MjMzMzk4NH19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo3OC42NDMxMTIxODI2MTcxOSwibGVmdCI6MTkuMDg4MzU2MDE4MDY2NDA2LCJ3aWR0aCI6MzYuNzI1MzExMjc5Mjk2ODc1LCJoZWlnaHQiOjEuOTAyNDQ0MjQzNDMxMDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo4MS44OTUxMzM5NzIxNjc5NywibGVmdCI6MTMuMjQ2ODU5NTUwNDc2MDc0LCJ3aWR0aCI6NDAuMzkxMzk1NTY4ODQ3NjU2LCJoZWlnaHQiOjEuOTA1MTU3MDg5MjMzMzk4NH19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo4NS4xNDcxNjMzOTExMTMyOCwibGVmdCI6MTUuNTgzNDU4OTAwNDUxNjYsIndpZHRoIjoxNy42MDIxNzg1NzM2MDg0LCJoZWlnaHQiOjEuOTA1MTU3MDg5MjMzMzk4NH19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo4OC4zOTkxOTI4MTAwNTg2LCJsZWZ0IjoxNy4zOTgyMTYyNDc1NTg1OTQsIndpZHRoIjo2NS4wMDczNDcxMDY5MzM2LCJoZWlnaHQiOjEuOTA1MTU3MDg5MjMzMzk4NH19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo5MC4wMjUyMDc1MTk1MzEyNSwibGVmdCI6MTkuMDg4MzU2MDE4MDY2NDA2LCJ3aWR0aCI6NjEuMDUxMDUyMDkzNTA1ODYsImhlaWdodCI6MS45MDI0NDQyNDM0MzEwOTEzfX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjEwLjM1MDQxNzEzNzE0NTk5NiwibGVmdCI6MjAuODQwODA1MDUzNzEwOTM4LCJ3aWR0aCI6NjYuMjc3MDUzODMzMDA3ODEsImhlaWdodCI6My41Mjg0NTUyNTc0MTU3NzE1fX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjE1LjQ5OTQ2Nzg0OTczMTQ0NSwibGVmdCI6MTcuMzM1OTA2OTgyNDIxODc1LCJ3aWR0aCI6MjMuNTE5NjE1MTczMzM5ODQ0LCJoZWlnaHQiOjEuOTA1MTQ4NjI1MzczODQwM319LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjoxOC43NTE1MDEwODMzNzQwMjMsImxlZnQiOjE5LjE1MDY2NTI4MzIwMzEyNSwid2lkdGgiOjY0LjQzODc2NjQ3OTQ5MjE5LCJoZWlnaHQiOjEuOTA1MTQ4NjI1MzczODQwM319LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjoyMC4zNzc1MTc3MDAxOTUzMTIsImxlZnQiOjIwLjg0MDgwNTA1MzcxMDkzOCwid2lkdGgiOjYxLjA3NDQyNDc0MzY1MjM0NCwiaGVpZ2h0IjozLjUyODQ1NDU0MjE2MDAzNH19LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjoyNS4yNTU1Njc1NTA2NTkxOCwibGVmdCI6MTkuMTUwNjY1MjgzMjAzMTI1LCJ3aWR0aCI6NjguMDIzNTU5NTcwMzEyNSwiaGVpZ2h0IjoxLjkwNTE0ODYyNTM3Mzg0MDN9fSx7InBhZ2VOdW1iZXIiOjEzLCJiYm94Ijp7InRvcCI6MjYuODgxNTgyMjYwMTMxODM2LCJsZWZ0IjoyMC44NDA4MDUwNTM3MTA5MzgsIndpZHRoIjo2Ni4wNzYyOTM5NDUzMTI1LCJoZWlnaHQiOjMuNTI4NDcwOTkzMDQxOTkyfX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjMyLjAzMDY1MTA5MjUyOTMsImxlZnQiOjE3LjMzNTkwNjk4MjQyMTg3NSwid2lkdGgiOjI0LjY3Nzk3NjYwODI3NjM2NywiaGVpZ2h0IjoxLjkwNTE0ODYyNTM3Mzg0MDN9fSx7InBhZ2VOdW1iZXIiOjEzLCJiYm94Ijp7InRvcCI6MzUuMjgyNjg0MzI2MTcxODc1LCJsZWZ0IjoxOS4xNTA2NjUyODMyMDMxMjUsIndpZHRoIjo2Mi40MTk3ODgzNjA1OTU3LCJoZWlnaHQiOjEuOTA1MTQ4NjI1MzczODQwM319LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjozNi45MDg2OTkwMzU2NDQ1MywibGVmdCI6MjAuODQwODA1MDUzNzEwOTM4LCJ3aWR0aCI6Ni42MzM5OTMxNDg4MDM3MTEsImhlaWdodCI6MS45MDI0NDAxOTAzMTUyNDY2fX1dfSwiY2h1bmtJZCI6ImJhNjRkNjBiLTk1NjItNDRiNS1iZWViLWNmNjUzZTI0YzczOCIsImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEzIn19XQ%3D%3D. Such stoichiometric imbalances often shift systems from phosphorus to nitrogen limitation, a transition that can paradoxically favor nitrogen-fixing cyanobacteria capable of exploiting excess phosphorus supplies (Nõges et al., 2020; Swann et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRpaW5hIiwiZmFtaWx5IjoiTsO1Z2VzIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hc2ltZSIsImZhbWlseSI6IkphbmF0aWFuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlJlZXQiLCJmYW1pbHkiOiJMYXVnYXN0ZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQZWV0ZXIiLCJmYW1pbHkiOiJOw7VnZXMifX1dLCJpZHMiOnsiaXNzbiI6WyIyMzUxLTk4OTQiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMTYvai5nZWNjby4yMDIwLmUwMTM2OSIsIm1hZyI6IjMxMDU0NDYwOTAiLCJvcGVuYWxleCI6IlczMTA1NDQ2MDkwIiwiZXhhIjoiaHR0cHM6Ly93d3cuc2NpZW5jZWRpcmVjdC5jb20vc2NpZW5jZS9hcnRpY2xlL3BpaS9TMjM1MTk4OTQyMDMwOTEwMCJ9LCJ0aXRsZSI6IlBvc3Qtc292aWV0IGNoYW5nZXMgaW4gbml0cm9nZW4gYW5kIHBob3NwaG9ydXMgc3RvaWNoaW9tZXRyeSBpbiB0d28gbGFyZ2Ugbm9uLXN0cmF0aWZpZWQgbGFrZXMgYW5kIHRoZSBpbXBhY3Qgb24gcGh5dG9wbGFua3RvbiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2ouZ2VjY28uMjAyMC5lMDEzNjkiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2ouZ2VjY28uMjAyMC5lMDEzNjkiLCJpc3N1ZWREYXRlIjoiMjAyMC0xMS0yMCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTEiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6Ikdsb2JhbCBFY29sb2d5IGFuZCBDb25zZXJ2YXRpb24iLCJ2b2x1bWUiOjI0LCJpbXBhY3RGYWN0b3IiOiIzLjU5In0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkVsc2V2aWVyIEJWIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo3MSwiY2l0ZWRCeUNvdW50IjoxNH19LCJxdW90ZSI6IlJhbmRvbSBGb3Jlc3QgYW5hbHlzaXMgaW5kaWNhdGVkIGFuIGltcG9ydGFudCByb2xlIG9mIGxpZ2h0IGxpbWl0YXRpb24gaW4gYm90aCBsYWtlcy4gQ29uc3RhbnRseSBsb3dlciBsZXZlbHMgb2YgTiBpbiB0aGUgZGVlcGVyIExha2UgUGVpcHNpIGZhdm91cmVkIE4yLWZpeGluZyBzcGVjaWVzLCB3aGljaCwgYXMgYSBwYXJhZG94LCBiZWNhbWUgUC1saW1pdGVkLiBBcyBjbGltYXRlIHdhcm1pbmcgcmVpbmZvcmNlcyBldXRyb3BoaWNhdGlvbiBwaGVub21lbmEgaW4gbGFrZXMgYnkgaW5jcmVhc2luZyBpbnRlcm5hbCBudXRyaWVudCBsb2FkaW5nIGFuZCBmYXZvdXJpbmcgYmxvb20tZm9ybWluZyBjeWFub2JhY3RlcmlhLCBtb3JlIHN0cmluZ2VudCBtZWFzdXJlcyB3b3VsZCBiZSBuZWVkZWQgdG8gZnVydGhlciBsaW1pdCBudXRyaWVudCBsb2FkcyAoZXNwZWNpYWxseSB0aGF0IG9mIFApIHRvIGxha2VzIHRocm91Z2ggaW1wcm92ZWQgd2FzdGV3YXRlciB0cmVhdG1lbnQgYW5kIGluY3JlYXNlZCBlZmZpY2llbmN5IG9mIGZlcnRpbGlzZXIgYXBwbGljYXRpb24uIExvbmctdGVybSBjaGFuZ2VzIGluIG51dHJpZW50IGxvYWRpbmcgYW5kIHN0b2ljaGlvbWV0cnkgaW4gd2F0ZXIgYm9kaWVzIHJlZmxlY3QgY2hhbmdlcyBpbiBhZ3JpY3VsdHVyYWwgcHJhY3RpY2VzIGFuZCB3YXN0ZXdhdGVyIHRyZWF0bWVudC4iLCJleHRlcm5hbENhY2hlSWQiOiI3M2Y5YjUzMS1hMDFjLTNiZWMtYWE4Zi1mMDU4MWVjZDkzZTUifSx7Im9yaWdpbiI6NSwid29yayI6eyJ0eXBlIjo4LCJzdWJ0eXBlIjp7Im9uZW9mS2luZCI6ImFydGljbGVUeXBlIiwiYXJ0aWNsZVR5cGUiOjF9LCJjb250cmlidXRvcnMiOlt7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNaWNhaCBNLiIsImZhbWlseSI6IlN3YW5uIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFsaWNpYSIsImZhbWlseSI6IkNvcnTDqXMifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQWxleGFuZGVyIEwuIiwiZmFtaWx5IjoiRm9ycmVzdCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJOaWNob2xhcyBULiIsImZhbWlseSI6IkZyYW1zdGVkIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlN0ZXZlbiIsImZhbWlseSI6IlNhZHJvIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikdlb2ZmcmV5IiwiZmFtaWx5IjoiU2NobGFkb3cifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW5nZWxhIERlIiwiZmFtaWx5IjoiUGFsbWHigJBEb3cifX1dLCJpZHMiOnsiaXNzbiI6WyIxMDE1LTE2MjEiLCIxNDIwLTkwNTUiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMDcvczAwMDI3LTAyNC0wMTA0NS0yIiwib3BlbmFsZXgiOiJXNDM5MjcxMzM5OSIsImV4YSI6Imh0dHBzOi8vbGluay5zcHJpbmdlci5jb20vYXJ0aWNsZS8xMC4xMDA3L3MwMDAyNy0wMjQtMDEwNDUtMj9lcnJvcj1jb29raWVzX25vdF9zdXBwb3J0ZWQmY29kZT04M2NmOGQ4OC1jOWEwLTRjZGQtODI1Ni0xYTg1ZTZhYmRhYzMifSwidGl0bGUiOiJJbnRlcm5hbCBwaG9zcGhvcnVzIGxvYWRpbmcgYWx0ZXJzIG51dHJpZW50IGxpbWl0YXRpb24gYW5kIGNvbnRyaWJ1dGVzIHRvIGN5YW5vYmFjdGVyaWFsIGJsb29tcyBpbiBhIHBvbHltaWN0aWMgbGFrZSIsImFic3RyYWN0IjoiQWJzdHJhY3QgQ2xlYXIgTGFrZSwgYSBtZWRpdW0tc2l6ZWQgaHlwZXJldXRyb3BoaWMsIHBvbHltaWN0aWMgbGFrZSBpbiBub3J0aGVybiBDYWxpZm9ybmlhLCBoYXMgaGFkIHJlY3VycmluZyBoYXJtZnVsIGN5YW5vYmFjdGVyaWEgYmxvb21zIChIQ0JzKSBmb3Igb3ZlciBhIGNlbnR1cnkgZGVzcGl0ZSByZWR1Y3Rpb25zIGluIGV4dGVybmFsIHBob3NwaG9ydXMgKFApIGxvYWRpbmdzLiBJbnRlcm5hbCBQIGxvYWRpbmcgY2FuIGFsdGVyIG51dHJpZW50IGF2YWlsYWJpbGl0eSBhbmQgbGltaXRhdGlvbiBzdXBwb3J0aW5nIEhDQnMgYnV0IGlzIHJhcmVseSBxdWFudGlmaWVkIG9yIGNvbXBhcmVkIHdpdGggZXh0ZXJuYWwgbG9hZHMuIFdlIGhhdmUgcXVhbnRpZmllZCBleHRlcm5hbCBQIGxvYWRzIGZyb20gMjAxOSB0byAyMDIxIGZvciB0aGUgdGhyZWUgbWFpbiB0cmlidXRhcmllcyAoYWNjb3VudGluZyBmb3IgNDYlIG9mIHRoZSBmbG93KSBhbmQgaW50ZXJuYWwgbG9hZGluZ3MgdXNpbmcgdHdvIG1ldGhvZHM6IGEgUCBtYXNzIGJhbGFuY2UgYW5kIG1vZGVsZWQgcmVsZWFzZSByYXRlcyBvZiBzb2x1YmxlIHJlYWN0aXZlIHBob3NwaG9ydXMgZnJvbSBveGljIGFuZCBhbm94aWMgc2VkaW1lbnRzLiBJbiBhZGRpdGlvbiwgd2UgY29tYmluZWQgaGlnaC1mcmVxdWVuY3kgaW4gc2l0dSBtZWFzdXJlbWVudHMgb2Ygd2F0ZXIgdGVtcGVyYXR1cmUgYW5kIGRpc3NvbHZlZCBveHlnZW4sIGRpc2NyZXRlIGdyYWIgc2FtcGxpbmcgZm9yIG51dHJpZW50IGNoZW1pc3RyeSwgYW5kIHJlbW90ZSBzZW5zaW5nIHRvIGV4cGxvcmUgdGhlIHBvdGVudGlhbCBkcml2ZXJzIG9mIHRoZSBvYnNlcnZlZCB2YXJpYWJpbGl0eSBhbmQgcHJvdmlkZSBhIGNvbXByZWhlbnNpdmUgdmlldyBvZiB0aGUgc3BhdGlvdGVtcG9yYWwgZHluYW1pY3Mgb2YgSENCcy4gQnkgdW5kZXJzdGFuZGluZyB0aGUgcmVsYXRpdmUgY29udHJpYnV0aW9uIG9mIGV4dGVybmFsIGFuZCBpbnRlcm5hbCBudXRyaWVudCBsb2FkaW5ncyBhbmQgdGhlIHJlbGF0aW9uc2hpcCBiZXR3ZWVuIGVudmlyb25tZW50YWwgcGFyYW1ldGVycyBhbmQgSENCcywgaW50ZXJhbm51YWwgYmxvb20gdmFyaWFiaWxpdHkgY2FuIGJlIGJldHRlciBwcmVkaWN0ZWQuIENvbXBhcmF0aXZlIGVzdGltYXRlcyBvZiBleHRlcm5hbCBhbmQgaW50ZXJuYWwgcGhvc3Bob3J1cyBsb2FkaW5nIGluZGljYXRlIHRoYXQgaW50ZXJuYWwgc291cmNlcyBhY2NvdW50ZWQgZm9yIDcw4oCTOTUlIG9mIHRoZSB0b3RhbCBQIGlucHV0IGludG8gdGhlIHN5c3RlbSBkdXJpbmcgdGhlIHN0dWR5IHBlcmlvZC4gQ29udHJhcnkgdG8gb3RoZXIgbGFrZXMsIHRoZSBpbnRlbnNpdHkgb2YgdGhlIHN1bW1lciBibG9vbSBzZWFzb24gd2FzIGNvcnJlbGF0ZWQgdG8gdGhlIHRpbWluZyBhbmQgZHVyYXRpb24gb2YgYW5veGlhIHJhdGhlciB0aGFuIHRoZSBtYWduaXR1ZGUgb2Ygc3ByaW5nIHJ1bm9mZi4gSW50ZXJuYWxseSByZWxlYXNlZCBQIHNoaWZ0ZWQgdGhlIHN5c3RlbSBmcm9tIHBob3NwaG9ydXMgdG8gbml0cm9nZW4gbGltaXRhdGlvbiBkdXJpbmcgdGhlIHN1bW1lciwgcG90ZW50aWFsbHkgZmF2b3JpbmcgdGhlIHByb2xpZmVyYXRpb24gb2Ygbml0cm9nZW4tZml4aW5nIGN5YW5vYmFjdGVyaWEuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjEwMDcvczAwMDI3LTAyNC0wMTA0NS0yIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2xpbmsuc3ByaW5nZXIuY29tL2NvbnRlbnQvcGRmLzEwLjEwMDcvczAwMDI3LTAyNC0wMTA0NS0yLnBkZiIsImlzc3VlZERhdGUiOiIyMDI0LTAzLTEyIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMSIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiQXF1YXRpYyBTY2llbmNlcyIsImlzc3VlIjoyLCJ2b2x1bWUiOjg2LCJpbXBhY3RGYWN0b3IiOiIyLjg0In0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlNwcmluZ2VyIFNjaWVuY2UrQnVzaW5lc3MgTWVkaWEifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjc1LCJjaXRlZEJ5Q291bnQiOjIxfX0sInF1b3RlIjoiQ29udHJhcnkgdG8gb3RoZXIgbGFrZXMsIHRoZSBpbnRlbnNpdHkgb2YgdGhlIHN1bW1lciBibG9vbSBzZWFzb24gd2FzIGNvcnJlbGF0ZWQgdG8gdGhlIHRpbWluZyBhbmQgZHVyYXRpb24gb2YgYW5veGlhIHJhdGhlciB0aGFuIHRoZSBtYWduaXR1ZGUgb2Ygc3ByaW5nIHJ1bm9mZi4gSW50ZXJuYWxseSByZWxlYXNlZCBQIHNoaWZ0ZWQgdGhlIHN5c3RlbSBmcm9tIHBob3NwaG9ydXMgdG8gbml0cm9nZW4gbGltaXRhdGlvbiBkdXJpbmcgdGhlIHN1bW1lciwgcG90ZW50aWFsbHkgZmF2b3JpbmcgdGhlIHByb2xpZmVyYXRpb24gb2Ygbml0cm9nZW4tZml4aW5nIGN5YW5vYmFjdGVyaWEuIC0gW0NhbGNpdW0gYW5kIHBob3NwaGF0ZSBtZXRhYm9saWMgZGlzb3JkZXJzXShodHRwczovL2xpbmsuc3ByaW5nZXIuY29tL3N1YmplY3RzL2NhbGNpdW0tYW5kLXBob3NwaGF0ZS1tZXRhYm9saWMtZGlzb3JkZXJzKSBFdXRyb3BoaWNhdGlvbiBwb3NlcyBhIGdyYXZlIHRocmVhdCB0byBsZW50aWMgZWNvc3lzdGVtcyBnbG9iYWxseSwgcHJvbW90aW5nIHRoZSBwcm9saWZlcmF0aW9uIG9mIGhhcm1mdWwgY3lhbm9iYWN0ZXJpYSBibG9vbXMgKEhDQnMpIHRoYXQgcmVkdWNlIGFxdWF0aWMgYmlvZGl2ZXJzaXR5LCB5aWVsZGluZyBpbmNyZWFzZWQgd2F0ZXIgdHJlYXRtZW50IGNvc3RzLCBhbmQgdGhyZWF0ZW5pbmcgaHVtYW4gaGVhbHRoIHRocm91Z2ggYmxvb20tcHJvZHVjZWQgY3lhbm90b3hpbnMgKEFyw6FveiBldCBhbC4iLCJleHRlcm5hbENhY2hlSWQiOiI0NTExZWJkNy1kZmQyLTM3NjAtYjcwNS01YTM0YzM5ZWU4YzMifV0%3D. Conversely, the proliferation of non-nitrogen-fixing, toxin-producing genera such as Microcystis and Planktothrix serves as a critical indicator of nitrogen over-enrichment, suggesting that continued external nitrogen loading remains essential for sustaining these specific harmful blooms (Paerl, 2018)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhhbnMgVy4iLCJmYW1pbHkiOiJQYWVybCJ9fV0sImlkcyI6eyJpc3NuIjpbXSwiaXNibiI6W10sImRvaSI6IjEwLjE3NjE1L3luNTUtYjQzOCIsIm9wZW5hbGV4IjoiVzQzMDcwNjM3MzkiLCJvcGVuQWNjZXNzSWQiOiIxODcxMjMwMTMwOTk2ODA2NTczIn0sInRpdGxlIjoiTWl0aWdhdGluZyB0b3hpYyBwbGFua3RvbmljIGN5YW5vYmFjdGVyaWFsIGJsb29tcyBpbiBhcXVhdGljIGVjb3N5c3RlbXMgZmFjaW5nIGluY3JlYXNpbmcgYW50aHJvcG9nZW5pYyBhbmQgY2xpbWF0aWMgcHJlc3N1cmVzIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjE3NjE1L3luNTUtYjQzOCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9jZHIubGliLnVuYy5lZHUvZG93bmxvYWRzL3doMjQ3MjkwZyIsImlzc3VlZERhdGUiOiIyMDE4LTAxLTAxIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0wNiIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiQ2Fyb2xpbmEgRGlnaXRhbCBSZXBvc2l0b3J5IChVbml2ZXJzaXR5IG9mIE5vcnRoIENhcm9saW5hIGF0IENoYXBlbCBIaWxsKSIsImltcGFjdEZhY3RvciI6IjAuMTAifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiVW5pdmVyc2l0eSBvZiBOb3J0aCBDYXJvbGluYSBhdCBDaGFwZWwgSGlsbCJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MCwiY2l0ZWRCeUNvdW50IjowfX0sInF1b3RlIjoiVGhpcyBsZWFkcyAgdG8gY29udGludWluZyBkZW1hbmQgZm9yIE4gc3VwcGxpZXMgdG8gc3VzdGFpbiBwcmltYXJ5IHByb2R1Y3Rpb24gYW5kIHByb21vdGUgZXV0cm9waGljYXRpb24gIFszOCwzOV0sICgyKSBhIGxlbmd0aHkgbGVnYWN5IG9mIFAgbG9hZGluZyB0aGF0IGlzIHN0b3JlZCBpbiBzZWRpbWVudHMsIHdoaWNoIGlzIHJlYWRpbHkgcmVjeWNsZWQgYW5kICBzdXBwb3J0cyBldXRyb3BoaWNhdGlvbiwgKDMpIE4yIGZpeGF0aW9uLCB3aGlsZSBwcm92aWRpbmcg4oCcbmV34oCdIE4sIGZhaWxzIHRvIG1lZXQgZWNvc3lzdGVtLXNjYWxlIE4gIHJlcXVpcmVtZW50cyBbNDAsNDFdLiBGcm9tIGEgbnV0cmllbnQgbWFuYWdlbWVudCBwZXJzcGVjdGl2ZSwgdGhpcyBtZWFucyB0aGF0IGNvbnRpbnVlZCBleHRlcm5hbCAgbG9hZGluZyBvZiBOIHBsYXlzIGEgY3JpdGljYWwgcm9sZSBpbiBhY2NlbGVyYXRpbmcgZXV0cm9waGljYXRpb24gYW5kIHByb21vdGluZyBhbGdhbCBibG9vbXMgWzM2XS4gIE9uZSB0cm91YmxpbmcgaW5kaWNhdG9yIG9mIE4tZHJpdmVuIGV1dHJvcGhpY2F0aW9uIGFuZCBDeWFub0hBQnMgaW4gZnJlc2h3YXRlciBzeXN0ZW1zIGlzICB0aGUgZ2VvZ3JhcGhpYyBleHBhbnNpb24gYW5kIGluY3JlYXNlIGZyZXF1ZW5jeSBvZiB0aGUgbm9uLU4yIGZpeGluZyB0b3hpbi1wcm9kdWNpbmcgZ2VudXMgIE1pY3JvY3lzdGlzIFs0Ml0sIGVzcGVjaWFsbHkgaW4gbGFyZ2UgbGFrZXMgKGUuZy4sIExha2UgRXJpZSwgQ2FuYWRhLVVTQSwgTGFrZSBLYXN1bWFndXJhLCBKYXBhbiwgTGFrZSAgT2tlZWNob2JlZSwgRmxvcmlkYSwgVVNBLCBMYWtlIFRhaWh1LCBDaGluYSkuIFRoaXMgZ2VudXMsIGFzIHdlbGwgYXMgYW5vdGhlciBwbGFua3RvbmljIG5vbi1OMiAgZml4aW5nIEN5YW5vSEFCIFBsYW5rdG90aHJpeCwgYW5kIHNldmVyYWwgYmVudGhpYyBhbmFsb2d1ZSBibG9vbSB0YXhhIChlLmcuLCBub24tTjIgZml4aW5nIEx5bmdieWEgIGFuZCBPc2NpbGxhdG9yaWEgc3BwLiksIHNlcnZlIGFzIGluZGljYXRvcnMgb2YgTi1vdmVyLWVucmljaG1lbnQgWzM2LDQyXS4gVGhlIHByb2xpZmVyYXRpb24gb2YgdGhlc2Ugbm9uLSBGaWd1cmUgNS4gR2xvYmFsIE4gYW5kIFAtYmFzZWQgZmVydGlsaXplciB1c2UgaW4gcmVsYXRpb24gdG8gdGhlIHdvcmxk4oCZcyBodW1hbiBwb3B1bGF0aW9uLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjc1LjQ2NTEyNjAzNzU5NzY2LCJsZWZ0IjoyMC4wNTgyOTQyOTYyNjQ2NSwid2lkdGgiOjY0LjAzNTY5NzkzNzAxMTcyLCJoZWlnaHQiOjEuMDg4NDIzNjA5NzMzNTgxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjc2LjkwODMwMjMwNzEyODksImxlZnQiOjE2Ljg0Njk1MjQzODM1NDQ5Miwid2lkdGgiOjY3LjIzODg2MTA4Mzk4NDM4LCJoZWlnaHQiOjEuMDg4NDIzNjA5NzMzNTgxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjc4LjM1MTQ3ODU3NjY2MDE2LCJsZWZ0IjoxNi44NDY5NTI0MzgzNTQ0OTIsIndpZHRoIjo2Ny4yNDcwNzAzMTI1LCJoZWlnaHQiOjEuMDg4NDIzNjA5NzMzNTgxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjc5Ljc5NDY1NDg0NjE5MTQsImxlZnQiOjE2Ljg0Njk1MjQzODM1NDQ5Miwid2lkdGgiOjY3LjI1MDAyMjg4ODE4MzYsImhlaWdodCI6MS4wODg1MTA2MzI1MTQ5NTM2fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6ODEuMjM3OTIyNjY4NDU3MDMsImxlZnQiOjE2Ljg0NzA0MDE3NjM5MTYsIndpZHRoIjo2Ny4yNDAwNTEyNjk1MzEyNSwiaGVpZ2h0IjoxLjA4ODQyMzYwOTczMzU4MTV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo4Mi42ODEwOTg5Mzc5ODgyOCwibGVmdCI6MTYuODQ3MDQwMTc2MzkxNiwid2lkdGgiOjY0LjcwODkxNTcxMDQ0OTIyLCJoZWlnaHQiOjEuMDg4NDIzNjA5NzMzNTgxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjg0LjEyNDI4MjgzNjkxNDA2LCJsZWZ0IjoyMC4wNTgzODM5NDE2NTAzOSwid2lkdGgiOjY0LjAyNzIwNjQyMDg5ODQ0LCJoZWlnaHQiOjEuMDg4NDIzNjA5NzMzNTgxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjg1LjU2MTAyNzUyNjg1NTQ3LCJsZWZ0IjoxNi44NDcwMzYzNjE2OTQzMzYsIndpZHRoIjo2Ny4yMzMzNjc5MTk5MjE4OCwiaGVpZ2h0IjoxLjA4ODQ1OTg0OTM1NzYwNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjg3LjAwNDI0MTk0MzM1OTM4LCJsZWZ0IjoxNi44NDcwMDc3NTE0NjQ4NDQsIndpZHRoIjo2Ny4yNDgyMjIzNTEwNzQyMiwiaGVpZ2h0IjoxLjA4ODQyMzYwOTczMzU4MTV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo4OC40NDc0MDI5NTQxMDE1NiwibGVmdCI6MTYuODQ3MDA3NzUxNDY0ODQ0LCJ3aWR0aCI6NjcuMDgzMTUyNzcwOTk2MSwiaGVpZ2h0IjoxLjA4ODQyMzYwOTczMzU4MTV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo4OS44OTA2MDIxMTE4MTY0LCJsZWZ0IjoxNi44NDcwMDc3NTE0NjQ4NDQsIndpZHRoIjo2Ny4yMzQ4OTM3OTg4MjgxMiwiaGVpZ2h0IjoxLjA4ODQyMzYwOTczMzU4MTV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo5MS4zMzM3NzgzODEzNDc2NiwibGVmdCI6MTYuODQ2OTg0ODYzMjgxMjUsIndpZHRoIjo2Ni44NjI0MTE0OTkwMjM0NCwiaGVpZ2h0IjoxLjA4ODQyMzYwOTczMzU4MTV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo1Ni44MTgxNjQ4MjU0Mzk0NSwibGVmdCI6MTYuNDI4NTE0NDgwNTkwODIsIndpZHRoIjo2LjM1NjQxNDMxODA4NDcxNywiaGVpZ2h0IjoxLjMxNjM3ODQ3NDIzNTUzNDd9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo1Ni44MDk2Mzg5NzcwNTA3OCwibGVmdCI6MjQuNDk0MzUyMzQwNjk4MjQyLCJ3aWR0aCI6NTkuMzQxMzAwOTY0MzU1NDcsImhlaWdodCI6MS4zMDU3MzIxMzEwMDQzMzM1fX1dfSwiY2h1bmtJZCI6Ijg3Y2ZiMTI4LWQxZGUtNGFhMS05NjhjLWJmNjdkMDRhZjM4NSIsImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjYifX1d. This dynamic is further complicated by internal nutrient loading, wherein a lengthy legacy of phosphorus stored in sediments is readily recycled to support ongoing eutrophication even when external inputs are reduced (Paerl, 2018)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhhbnMgVy4iLCJmYW1pbHkiOiJQYWVybCJ9fV0sImlkcyI6eyJpc3NuIjpbXSwiaXNibiI6W10sImRvaSI6IjEwLjE3NjE1L3luNTUtYjQzOCIsIm9wZW5hbGV4IjoiVzQzMDcwNjM3MzkiLCJvcGVuQWNjZXNzSWQiOiIxODcxMjMwMTMwOTk2ODA2NTczIn0sInRpdGxlIjoiTWl0aWdhdGluZyB0b3hpYyBwbGFua3RvbmljIGN5YW5vYmFjdGVyaWFsIGJsb29tcyBpbiBhcXVhdGljIGVjb3N5c3RlbXMgZmFjaW5nIGluY3JlYXNpbmcgYW50aHJvcG9nZW5pYyBhbmQgY2xpbWF0aWMgcHJlc3N1cmVzIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjE3NjE1L3luNTUtYjQzOCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9jZHIubGliLnVuYy5lZHUvZG93bmxvYWRzL3doMjQ3MjkwZyIsImlzc3VlZERhdGUiOiIyMDE4LTAxLTAxIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0wNiIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiQ2Fyb2xpbmEgRGlnaXRhbCBSZXBvc2l0b3J5IChVbml2ZXJzaXR5IG9mIE5vcnRoIENhcm9saW5hIGF0IENoYXBlbCBIaWxsKSIsImltcGFjdEZhY3RvciI6IjAuMTAifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiVW5pdmVyc2l0eSBvZiBOb3J0aCBDYXJvbGluYSBhdCBDaGFwZWwgSGlsbCJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MCwiY2l0ZWRCeUNvdW50IjowfX0sInF1b3RlIjoiVGhpcyBsZWFkcyB0byBjb250aW51aW5nIGRlbWFuZCBmb3IgTiBzdXBwbGllcyB0byBzdXN0YWluIHByaW1hcnkgcHJvZHVjdGlvbiBhbmQgcHJvbW90ZSBldXRyb3BoaWNhdGlvbiBbMzgsMzldLCAoMikgYSBsZW5ndGh5IGxlZ2FjeSBvZiBQIGxvYWRpbmcgdGhhdCBpcyBzdG9yZWQgaW4gc2VkaW1lbnRzLCB3aGljaCBpcyByZWFkaWx5IHJlY3ljbGVkIGFuZCBzdXBwb3J0cyBldXRyb3BoaWNhdGlvbiwgKDMpIE4yIGZpeGF0aW9uLCB3aGlsZSBwcm92aWRpbmcg4oCcbmV34oCdIE4sIGZhaWxzIHRvIG1lZXQgZWNvc3lzdGVtLXNjYWxlIE4gcmVxdWlyZW1lbnRzIFs0MCw0MV0uIEZyb20gYSBudXRyaWVudCBtYW5hZ2VtZW50IHBlcnNwZWN0aXZlLCB0aGlzIG1lYW5zIHRoYXQgY29udGludWVkIGV4dGVybmFsIGxvYWRpbmcgb2YgTiBwbGF5cyBhIGNyaXRpY2FsIHJvbGUgaW4gYWNjZWxlcmF0aW5nIGV1dHJvcGhpY2F0aW9uIGFuZCBwcm9tb3RpbmcgYWxnYWwgYmxvb21zIFszNl0uIE9uZSB0cm91YmxpbmcgaW5kaWNhdG9yIG9mIE4tZHJpdmVuIGV1dHJvcGhpY2F0aW9uIGFuZCBDeWFub0hBQnMgaW4gZnJlc2h3YXRlciBzeXN0ZW1zIGlzIHRoZSBnZW9ncmFwaGljIGV4cGFuc2lvbiBhbmQgaW5jcmVhc2UgZnJlcXVlbmN5IG9mIHRoZSBub24tTjIgZml4aW5nIHRveGluLXByb2R1Y2luZyBnZW51cyBNaWNyb2N5c3RpcyBbNDJdLCBlc3BlY2lhbGx5IGluIGxhcmdlIGxha2VzIChlLmcuLCBMYWtlIEVyaWUsIENhbmFkYS1VU0EsIExha2UgS2FzdW1hZ3VyYSwgSmFwYW4sIExha2UgT2tlZWNob2JlZSwgRmxvcmlkYSwgVVNBLCBMYWtlIFRhaWh1LCBDaGluYSkuIFRoaXMgZ2VudXMsIGFzIHdlbGwgYXMgYW5vdGhlciBwbGFua3RvbmljIG5vbi1OMiBmaXhpbmcgQ3lhbm9IQUIgUGxhbmt0b3Rocml4LCBhbmQgc2V2ZXJhbCBiZW50aGljIGFuYWxvZ3VlIGJsb29tIHRheGEgKGUuZy4sIG5vbi1OMiBmaXhpbmcgTHluZ2J5YSBhbmQgT3NjaWxsYXRvcmlhIHNwcC4pLCBzZXJ2ZSBhcyBpbmRpY2F0b3JzIG9mIE4tb3Zlci1lbnJpY2htZW50IFszNiw0Ml0uIFRoZSBwcm9saWZlcmF0aW9uIG9mIHRoZXNlIG5vbi1OMiBmaXhpbmcgQ3lhbm9IQUJzIGlzIGFuIGFkZGl0aW9uYWwgaW5kaWNhdG9yIHRoZSBOIGlucHV0IHJlZHVjdGlvbnMgYXJlIG5lZWRlZCB0byByZXZlcnNlIHRoaXMgdHJlbmQuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NzcuNDQ2MjUwOTE1NTI3MzQsImxlZnQiOjE2LjQyODUxNDQ4MDU5MDgyLCJ3aWR0aCI6NzAuNzE0NzIxNjc5Njg3NSwiaGVpZ2h0IjoxLjQ1MDc5NjQ4NDk0NzIwNDZ9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo3OS4wNTA5Nzk2MTQyNTc4MSwibGVmdCI6MTIuODU3MDYxMzg2MTA4Mzk4LCJ3aWR0aCI6NzQuNDk0OTg3NDg3NzkyOTcsImhlaWdodCI6MS40NTA3OTY0ODQ5NDcyMDQ2fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6ODAuNjU1NTg2MjQyNjc1NzgsImxlZnQiOjEyLjgwMTk2MTg5ODgwMzcxMSwid2lkdGgiOjc0LjM0MDAxOTIyNjA3NDIyLCJoZWlnaHQiOjEuNDUwNzk2NDg0OTQ3MjA0Nn19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjgyLjI2MDE4NTI0MTY5OTIyLCJsZWZ0IjoxMi44NTcwNjEzODYxMDgzOTgsIndpZHRoIjo3NC4yODUxODY3Njc1NzgxMiwiaGVpZ2h0IjoxLjY2Njk3NzY0Mzk2NjY3NDh9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo4My44NjQ5MDYzMTEwMzUxNiwibGVmdCI6MTIuODU3MDYxMzg2MTA4Mzk4LCJ3aWR0aCI6NzQuMjg1MDE4OTIwODk4NDQsImhlaWdodCI6MS40NTA3OTY0ODQ5NDcyMDQ2fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6ODUuNDY5NTEyOTM5NDUzMTIsImxlZnQiOjEyLjg1NzA2MTM4NjEwODM5OCwid2lkdGgiOjcxLjEwNDk4ODA5ODE0NDUzLCJoZWlnaHQiOjEuNDUwNzk2NDg0OTQ3MjA0Nn19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjg3LjA3NDIzNDAwODc4OTA2LCJsZWZ0IjoxNi40Mjg1MTQ0ODA1OTA4Miwid2lkdGgiOjcwLjcxNDE3MjM2MzI4MTI1LCJoZWlnaHQiOjEuNDUwNzk2NDg0OTQ3MjA0Nn19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjg4LjY3ODg0MDYzNzIwNzAzLCJsZWZ0IjoxMi44NTcwNjEzODYxMDgzOTgsIndpZHRoIjo3NC4yODYwMjYwMDA5NzY1NiwiaGVpZ2h0IjoxLjY2Njk3NzY0Mzk2NjY3NDh9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo5MC4yODM0Mzk2MzYyMzA0NywibGVmdCI6MTIuODU3MDYxMzg2MTA4Mzk4LCJ3aWR0aCI6NzQuMjg1NDE1NjQ5NDE0MDYsImhlaWdodCI6MS40NzY4MzA0ODI0ODI5MTAyfX0seyJwYWdlTnVtYmVyIjo2LCJiYm94Ijp7InRvcCI6MTAuNTA2NTEzNTk1NTgxMDU1LCJsZWZ0IjoxMi44NTcwNjEzODYxMDgzOTgsIndpZHRoIjo3NC4yMDIwMTg3Mzc3OTI5NywiaGVpZ2h0IjoxLjU4NjYwODQwOTg4MTU5MTh9fSx7InBhZ2VOdW1iZXIiOjYsImJib3giOnsidG9wIjoxMi4xMTEyMzc1MjU5Mzk5NDEsImxlZnQiOjEyLjg1NzA2MTM4NjEwODM5OCwid2lkdGgiOjc0LjI4NTY3NTA0ODgyODEyLCJoZWlnaHQiOjEuNjY2OTgxMzM5NDU0NjUwOX19LHsicGFnZU51bWJlciI6NiwiYmJveCI6eyJ0b3AiOjEzLjcxNTgzNzQ3ODYzNzY5NSwibGVmdCI6MTIuODU3MDYxMzg2MTA4Mzk4LCJ3aWR0aCI6NzQuMjg1OTcyNTk1MjE0ODQsImhlaWdodCI6MS40NzY4MzQxNzc5NzA4ODYyfX0seyJwYWdlTnVtYmVyIjo2LCJiYm94Ijp7InRvcCI6MTUuMzIwNTYxNDA4OTk2NTgyLCJsZWZ0IjoxMi44NTcwNjEzODYxMDgzOTgsIndpZHRoIjo3NC4yODUxMDI4NDQyMzgyOCwiaGVpZ2h0IjoxLjY2Njg2NTM0ODgxNTkxOH19LHsicGFnZU51bWJlciI6NiwiYmJveCI6eyJ0b3AiOjE2LjkyNTE2MzI2OTA0Mjk3LCJsZWZ0IjoxMi44NTcwNjEzODYxMDgzOTgsIndpZHRoIjo3NC4yODUzNTQ2MTQyNTc4MSwiaGVpZ2h0IjoxLjY2Njk4MTMzOTQ1NDY1MDl9fV19LCJjaHVua0lkIjoiNzdlNTQwOTYtYzMwMi00NWZlLTg3OWQtNTc4MDc3YmQ5Y2MyIiwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNiJ9fV0%3D. Specifically, the mineralization of organic matter under anoxic conditions at the sediment-water interface facilitates the release of bioavailable phosphorus, which diffuses into the overlying water column to sustain primary production (Haxthausen, 2021)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkthcmwgUmFzbXVzIEF1Z3VzdCIsImZhbWlseSI6IkhheHRoYXVzZW4ifX1dLCJpZHMiOnsiaXNzbiI6W10sImlzYm4iOltdLCJtYWciOiIzMTc3MTEzMjA3Iiwib3BlbmFsZXgiOiJXMzE3NzExMzIwNyIsIm9wZW5BY2Nlc3NJZCI6IjEyMTQzNzIxNzkzMzgxMDMyOTc4In0sInRpdGxlIjoiU2VkaW1lbnQgbWljcm9iaWFsIGZ1ZWwgY2VsbHMgZm9yIGxha2UgcmVzdG9yYXRpb24iLCJhYnN0cmFjdCI6IkV1dHJvcGhpY2F0aW9uIG9mIGxha2VzIGlzIHViaXF1aXRvdXMsIGFzIDMwLTQwICUgb2YgdGhlIHdvcmxk4oCZcyBsYWtlcyBhcmUgZXV0cm9waGljLCBhbmQgY29uZGl0aW9ucyBjb250aW51b3VzbHkgZGV0ZXJpb3JhdGluZy4gRXV0cm9waGljYXRpb24gaXMgYSBjb25kaXRpb24gb2Ygb3ZlcmZlcnRpbGl6YXRpb24sIHdoZXJlIGFsbCByZXF1aXJlbWVudHMgZm9yIHJhcGlkIGdyb3d0aCBvZiBhbGdhZSBhcmUgbWV0LCByZXN1bHRpbmcgaW4gYWxnYWwgYmxvb21zLiBUaGlzIGNhdXNlcyBhIHNlbGYtc3VzdGFpbmVkLCB2aWNpb3VzIGN5Y2xlIHdoZXJlIHBvb3IgZWNvbG9naWNhbCBjb25kaXRpb25zIGFyZSBtYWludGFpbmVkIHVudGlsIG51dHJpZW50cyBhcmUgbG9zdCBmcm9tIHRoZSBsYWtlIG92ZXIgbWFueSB5ZWFycy4gTGFrZSByZXN0b3JhdGlvbiBzZWVrcyB0byBicmVhayB0aGlzIHZpY2lvdXMgY3ljbGUuIEluYWRlcXVhdGUgYXZhaWxhYmlsaXR5IG9mIGFueSBzaW5nbGUgbnV0cmllbnQgaW4gd2lsbCBsaW1pdCBhbGdhbCBncm93dGgsIHR5cGljYWxseSBwaG9zcGhvcnVzIGlzIHRhcmdldGVkLiBUaGUgc2NvcGUgb2YgdGhpcyBQaEQgdGhlc2lzIHdhcyB0byBkZXZlbG9wLCB0ZXN0IGFuZCBvcHRpbWl6ZSBzZWRpbWVudCBtaWNyb2JpYWwgZnVlbCBjZWxscyAoU01GQykgYXMgYSBub3ZlbCBtZXRob2QgZm9yIGxha2UgcmVzdG9yYXRpb24uIFRoZSB0aGVzaXMgc3BhbnMgdGhlIHR3byBtYWluIHRvcGljcyBvZiBTTUZDcyBhbmQgbGFrZSByZXN0b3JhdGlvbiB0ZWNobmlxdWVzLiBMYWtlIHJlc3RvcmF0aW9uIHRlY2huaXF1ZXMgYXJlIHJldmlld2VkLCBwcm92aWRpbmcgdW5kZXJzdGFuZGluZyBvZiB0aGUgdW5kZXJseWluZyBtZWNoYW5pc21zLiBTTUZDcyBhcmUgYXBwcm9hY2hlZCBmcm9tIGFuIGVuZ2luZWVyaW5nIHBlcnNwZWN0aXZlIGFuZCBhIGZvY3VzIG9uIGxha2UgcmVtZWRpYXRpb24uIFRoZSBzcGVjaWZpYyBvYmplY3RpdmVzIHdlcmUgdG8gdGVzdCBhIGxvdy1jb3N0IGVsZWN0cm9kZSBtYXRlcmlhbCBmb3IgaXRzIGR1cmFiaWxpdHksIHByYWN0aWNhbGl0eSwgYW5kIHBlcmZvcm1hbmNlLiBQZXJmb3JtYW5jZSB3YXMgYXNzZXNzZWQgYXMgdGhlIGFiaWxpdHkgdG8gbGltaXQgcGhvc3Bob3J1cyByZWxlYXNlIGZyb20gc2VkaW1lbnRzLCBpbmhpYml0IG1ldGhhbmUgcmVsZWFzZSwgYW5kIHRoZSBhZGRlZCBiZW5lZml0cyBvZiBkZWdyYWRpbmcgcG9sbHV0YW50cywgYW5kIGdlbmVyYXRpbmcgZWxlY3RyaWNpdHkuIFByb29mIG9mIGNvbmNlcHQgd2FzIHNob3duIGluIGxhYm9yYXRvcnkgcmVhY3RvcnMgKDEgTCksIHRoZSBlZmZlY3Qgb2YgdXBzY2FsaW5nIHRvIDIwMCBMIGluIG5hdHVyYWwgZW52aXJvbm1lbnQgd2FzIHN0dWRpZWQgaW4gb3V0ZG9vciBtZXNvY29zbXMsIGFuZCB0aGVuIHRlc3RlZCBhdCBwaWxvdCBzY2FsZSBpbiBhIGV1dHJvcGhpYyBmcmVzaHdhdGVyIGxha2UuIFBpbG90IGluc3RhbGxhdGlvbnMgd2VyZSB0aGUgbGFyZ2VzdCBpbiB0aGUgd29ybGQgb2YgdGhlaXIga2luZCBhbmQgd2VyZSBvcGVyYXRlZCBmb3IgMiB5ZWFycy4gU3RhaW5sZXNzIHN0ZWVsIHdhcyBzZWxlY3RlZCBmb3IgdGVzdGluZyBhcyBlbGVjdHJvZGUgbWF0ZXJpYWwgZHVlIHRvIGl0cyBsb3cgY29zdCwgZHVyYWJpbGl0eSwgYW5kIGF2YWlsYWJpbGl0eS4gUGFpcnMgb2YgZWxlY3Ryb2RlcyB3ZXJlIGVtYmVkZGVkIGluIHRoZSBzZWRpbWVudCAodGhpcyBpcyBjYWxsZWQgdGhlIGFub2RlKSBhbmQgc3VzcGVuZGVkIGluIHRoZSBvdmVybHlpbmcgd2F0ZXIgKHRoaXMgaXMgY2FsbGVkIHRoZSBjYXRob2RlKS4gVGhlIHR3byBlbGVjdHJvZGVzIHdlcmUgY29ubmVjdGVkIGFjcm9zcyBhbiBleHRlcm5hbCByZXNpc3RvciB3aXRoIGVsZWN0cmljYWwgd2lyZXMuIE1pY3JvYmVzIGluIHRoZSBzZWRpbWVudCB1c2UgdGhlIGFub2RlIHBhaXIgYXMgZWxlY3Ryb24gYWNjZXB0b3IsIGVuYWJsaW5nIHJlc3BpcmF0aW9uIGluIHRoZSBhbm94aWMgc2VkaW1lbnQuIEVsZWN0cm9ucyB0cmF2ZWwgZnJvbSB0aGUgc2VkaW1lbnQgdmlhIHRoZSBlbGVjdHJpY2FsIGNvbm5lY3RvciB0byB0aGUgb3Zlcmx5aW5nIGNhdGhvZGUgd2hlcmUgb3h5Z2VuIGluIHRoZSB3YXRlciBpcyB0aGUgdGVybWluYWwgZWxlY3Ryb24gYWNjZXB0b3IgY29tcGxldGluZyB0aGUgY2lyY3VpdC4gQXMgYW4gYWRkZWQgYmVuZWZpdCwgZGVncmFkYXRpb24gb2YgcG9sbHV0YW50cyB1c2luZyBTTUZDIHdhcyB0ZXN0ZWQgaW4gbWFyaW5lIGhhcmJvdXIgc2VkaW1lbnRzLCBidXQgbm8gY29uY2x1c2l2ZSByZXN1bHRzIG9idGFpbmVkLiBMYWJvcmF0b3J5IHJlYWN0b3JzIHdlcmUgb3BlcmF0ZWQgZm9yIG1vcmUgdGhhbiAyMDAgZGF5cyB1c2luZyBwZXRyb2xldW0gYXMgc29sZSBjYXJib24gc291cmNlLCBjb25maXJtaW5nIG9pbCBkZWdyYWRhdGlvbiBieSBTTUZDcy4gQSByZW1vdmFsIHJhdGUgb2YgMC45IGcgcGV0cm9sZXVtIG0tMyBkLTEgcmVhY3RvciB2b2x1bWUgd2FzIGVzdGltYXRlZCBmcm9tIG1lYXN1cmVkIGVsZWN0cmljYWwgb3V0cHV0LiBUaGlzIHdhcyBkb25lIGJ5IGFzc3VtcHRpb24gb2YgdGhlIGNoZW1pY2FsIHJlYWN0aW9uIG9jY3VycmluZywgZnJvbSB0aGlzIHRoZSBudW1iZXIgb2YgZWxlY3Ryb25zIHRyYW5zZmVycmVkIGZyb20gb2lsIHRvIHdhdGVyIHdhcyBrbm93biBhbmQgY291bGQgYmUgY29ycmVsYXRlZCB0byB0aGUgbWVhc3VyZWQgZmxvdyBvZiBlbGVjdHJvbnMgKGN1cnJlbnQpIHRocm91Z2ggdGhlIGV4dGVybmFsIGNpcmN1aXQuIFN0YWlubGVzcyBzdGVlbCBlbGVjdHJvZGVzIHdlcmUgb3BlcmF0ZWQgaW4gZnJlc2h3YXRlciBsYWtlIGVudmlyb25tZW50cyBmb3IgbW9yZSB0aGFuIDQgeWVhcnMgd2l0aCBtaW5pbWFsIGRhbWFnZSB0byB0aGUgbWF0ZXJpYWwuIFRoZSBlbGVjdHJvZGUgaXMgdGhlIG1haW4gbWF0ZXJpYWwgY29zdCBpbiBzdWNoIHN5c3RlbXMuIEhvd2V2ZXIsIEFJU0kzMTYgc3RhaW5sZXNzIHN0ZWVsIHNpbWlsYXIgdG8gd2hhdCB3YXMgdXNlZCBpbiB0aGlzIHRoZXNpcyB3YXMgYXQgdGhlIHRpbWUgb2Ygd3JpdGluZyBhdmFpbGFibGUgYXQgYnVsayBjb3N0IG9mIDAuMDkxIOKCrCBtLTIsIGNvcnJlc3BvbmRpbmcgdG8gOTEwIOKCrCBhY3JlLTEsIHNpZ25pZmljYW50bHkgbG93ZXIgdGhhbiBvdGhlciBjb21tb24gbWF0ZXJpYWxzLiBDYXRob2RlcyBhdCBsYXJnZSBzY2FsZSBjb3VsZCBiZSBtb3ZlZCBob3Jpem9udGFsbHkgYW5kIHZlcnRpY2FsbHkgd2l0aCBtaW5vciBpbXBhY3QgdG8gcGVyZm9ybWFuY2UuIFBpbG90IHNjYWxlIFNNRkNzIGdlbmVyYXRlZCAwLjM4MyBtVyBtLTIgc3VzdGFpbmVkIGFuZCAyNy40IG1XIG0tMiBwZWFrIG91dHB1dC4gVGhlIHN5c3RlbSB3YXMgbGltaXRlZCBieSB0aGUgY2F0aG9kZSBzdXJmYWNlIGFyZWEsIG1lYW5pbmcgbXVsdGktbGF5ZXIgY2F0aG9kZXMgc2hvdWxkIGJlIHRlc3RlZCBpbiB0aGUgZnV0dXJlIGlmIGVsZWN0cmljYWwgcG93ZXIgcHJvZHVjdGlvbiBpcyB0aGUgZ29hbC4gVGhlIGNhdGhvZGUgc2hvdWxkIGJlIHBsYWNlZCBhdCBhIGRlcHRoIHRoYXQgZW5zdXJlcyBoaWdoIGRpc3NvbHZlZCBveHlnZW4gY29uY2VudHJhdGlvbnMgdG8gYXZvaWQgdGhlIHN5c3RlbSB0byBiZSBsaW1pdGVkLiBIb3dldmVyLCB0b28gaGlnaCBlbGVjdHJpY2l0eSBnZW5lcmF0aW9uIHdhcyBmb3VuZCBjb3VudGVycHJvZHVjdGl2ZSBmb3IgcmVtZWRpYXRpb24gcHVycG9zZXMgYmVjYXVzZSB0aGlzIHdvdWxkIGluY3JlYXNlIHNlZGltZW50IHR1cm5vdmVyIHJhdGUsIHJlbGVhc2luZyBwb2xsdXRhbnRzIHN0b3JlZCBpbiB0aGUgc2VkaW1lbnRzLiBQIGJpbmRpbmcgd2FzIG5vdCBmb3VuZCB0byByZXF1aXJlIGhpZ2ggdHVybm92ZXIgb2Ygc2VkaW1lbnQgb3JnYW5pYyBtYXR0ZXIuIFRoZSB0dXJub3ZlciBjYW4gYmUgY29udHJvbGxlZCBieSBjaGFuZ2luZyB0aGUgZXh0ZXJuYWwgcmVzaXN0YW5jZS4gSGlnaCByZXNpc3RhbmNlIGxpbWl0cyB0aGUgZWxlY3RyaWNhbCBwZXJmb3JtYW5jZSwgbG93IHJlc2lzdGFuY2UgaW5jcmVhc2VzIHNlZGltZW50IHR1cm5vdmVyIGFuZCByaXNrcyBkZXN0YWJpbGl6aW5nIHRoZSBzeXN0ZW0uIFRvIGFjaGlldmUgcmVkdWNlZCBjb25jZW50cmF0aW9ucyBvZiBwaG9zcGhhdGUgaW4gdGhlIHdhdGVyIGNvbHVtbiwgYW5vZGVzIG11c3QgYmUgcGxhY2VkIGluIHRoZSBzZWRpbWVudCBzdXJmYWNlIHNpbmNlIGEgZmV3IG1pbGxpbWV0cmVzIG9mIHNlZGltZW50IG1heSBjb250YWluIGFuZCByZWxlYXNlIGVub3VnaCBQIHRvIHN1c3RhaW4gZXV0cm9waGljYXRpb24uIFBob3NwaGF0ZSB3YXMgbWVhc3VyZWQgaW4gdGhlIHdhdGVyIGNvbHVtbiBuZWFyIHRoZSBzZWRpbWVudCwgaW4gc3lzdGVtcyB3aXRoIGFuZCB3aXRob3V0IGVsZWN0cm9kZXMuIFBob3NwaGF0ZSBjb25jZW50cmF0aW9ucyB3ZXJlIHN1Y2Nlc3NmdWxseSByZWR1Y2VkIGJ5IHVwIHRvIDk1ICUgaW4gMi1jaGFtYmVyIGxhYm9yYXRvcnkgcmVhY3RvcnMsIGFuZCBieSB1cCB0byA5OCAlIGluIHRoZSBwaWxvdCBpbnN0YWxsYXRpb24uIEluIHRoZSBwaWxvdCBleHBlcmltZW50LCBwaG9zcGhhdGUgY29uY2VudHJhdGlvbnMgYWJvdmUgZWxlY3Ryb2RlcyB3ZXJlIGxvd2VyIGluIGFsbCBtZWFzdXJlbWVudHMsIGNvbXBhcmVkIHRvIGNvbnRyb2xzLiBTZWRpbWVudCByZWRveCBwb3RlbnRpYWxzIGluY3JlYXNlZCBmcm9tIC00MDAgbVYgdnMgc3RhbmRhcmQgaHlkcm9nZW4gZWxlY3Ryb2RlIChTSEUpIHRvIC0xNzUgbVYgdnMgU0hFLCBjb3JyZXNwb25kaW5nIHRvIGEgbW92ZSBmcm9tIGEgbWV0aGFub2dlbmV0aWMgZG9tYWluIHRvIHN1bHBodXIgcmVkdWNpbmcuIEFuYWx5c2lzIG9mIHRoZSBtaWNyb2JpYWwgY29tbXVuaXR5IGluZGljYXRlZCBlbmhhbmNlZCBzdWxwaHVyIGN5Y2xpbmcgd2hlcmUgZWxlY3Ryb2RlcyBveGlkaXplIHN1bHBoaWRlIHdoaWNoIHRoZSBiYWN0ZXJpYSBhZ2FpbiByZWR1Y2UgdG8gb2J0YWluIGVuZXJneS4gSXQgd2FzIGh5cG90aGVzaXNlZCB0aGF0IFNNRkMgcHJvbW90ZSBoaWdoZXIgcmVkb3ggcG90ZW50aWFscyBieSBmYWNpbGl0YXRpbmcgb3hpZGF0aW9uIG9mIHN1bHBodXIgYW5kIGlyb24gc3BlY2llcywgaW5oaWJpdGluZyBIMlMgZm9ybWF0aW9uIGFuZCBGZSBzY2F2ZW5naW5nIGJ5IEZlUyBmb3JtYXRpb24sIHRodXMgYWxsb3dpbmcgZm9yIFAgYmluZGluZy4gQSBsb3dlciByZWxhdGl2ZSBhYnVuZGFuY2Ugb2YgbWV0aGFub2dlbnMgb24gZWxlY3Ryb2RlcyBjb21wYXJlZCB0byBjb250cm9scyBzdWdnZXN0cyBzdXBwcmVzc2lvbiBvciBvdXRjb21wZXRpbmcgb2YgbWV0aGFub2dlbnMuUXVhbnRpZmljYXRpb24gb2YgbWV0aGFuZSBwcm9kdWN0aW9uIHdhcyBkb25lIG9uIHRoZSBiYXNpcyBvZiBhIENINDpDTzIgcmF0aW8gdG8gYWxsb3cgY29tcGFyYWJpbGl0eSBiZXR3ZWVuIHN5c3RlbXMuIE1ldGhhbmUgZW1pc3Npb24gd2FzIHJlZHVjZWQgYnkgdXAgdG8gOTkgJSBhbmQgbWFpbnRhaW5lZCBpbmhpYml0aW9uIGZvciBzZXZlcmFsIG1vbnRocyBpbiBsYWJvcmF0b3J5IHN5c3RlbXMgd2l0aCBlbGVjdHJvZGVzLiBJbiB0aGUgcGlsb3QgaW5zdGFsbGF0aW9ucyBubyBnYXMgc2FtcGxlcyBjb3VsZCBiZSBjb2xsZWN0ZWQsIGFuZCBubyBDTzI6Q0g0IHJhdGlvcyB3ZXJlIG9idGFpbmVkLiBBZXJhdGlvbiBpcyB0aGUgY3VycmVudCByZXN0b3JhdGlvbiB0ZWNobmlxdWUgdGhhdCBtb3N0IGNsb3NlbHkgcmVzZW1ibGVzIHRoZSBwcm9wb3NlZCBTTUZDIHRlY2huaXF1ZS4gQWVyYXRpb24gaGFzIHByZXZpb3VzbHkgZmFpbGVkIHRvIHJlZHVjZSBQIHJlbGVhc2UgaW4gc2V2ZXJhbCBzZXBhcmF0ZSBpbnZlc3RpZ2F0aW9ucy4gU01GQ3MgYXMgYSByZW1lZGlhdGlvbiB0b29sIHdhcyBmb3VuZCB0byBvZmZlciBzZXZlcmFsIGJlbmVmaXRzIG92ZXIgY29udmVudGlvbmFsIG1ldGhvZHM6IFNNRkNzIG9mZmVycyBjb250aW51b3VzIGVmZmVjdCwgd2hpbGUgb3RoZXIgdGVjaG5pcXVlcyByZXF1aXJlIGNvbnRpbnVvdXMgb3IgcmVwZWF0ZWQgdHJlYXRtZW50LiBTTUZDcyBjYW4gYmUgY29udHJvbGxlZCBpbiByZWFsLXRpbWUgYnkgYWx0ZXJpbmcgdGhlIGV4dGVybmFsIHJlc2lzdG9yIGFuZCBjYW4gYmUgZWFzaWx5IG1vbml0b3JlZCBieSBtZWFzdXJpbmcgdGhlIHByb2R1Y2VkIGVsZWN0cmljYWwgb3V0cHV0LiBJZiBpbXBsZW1lbnRlZCBhdCBsYXJnZSBzY2FsZSBpdCBpcyBleHBlY3RlZCB0aGF0IHRoZSBlbGVjdHJvZGUgZmllbGRzIHdpbGwgaGVscCBtYWludGFpbiBsb3cgcGhvc3BoYXRlIGNvbmNlbnRyYXRpb25zIGluIHRoZSB3YXRlciBmb3IgZGVjYWRlcyB3aXRoIG1pbmltYWwgaW50ZXJ2ZW50aW9uLCBhbmQgYWZ0ZXIgc29tZSB0aW1lLCBiZW50aGljIHBsYW50cyBhcmUgZXhwZWN0ZWQgdG8gcmV0dXJuLCB0aHVzIG1haW50YWluaW5nIHRoZSBiYWxhbmNlIGhlbmNlIGZvcnRoLiIsInVybCI6Imh0dHBzOi8vbG9jYWwuZm9yc2tuaW5nc3BvcnRhbC5kay9sb2NhbC9ka2ktY2dpL3dzL2NyaXMtbGluaz9zcmM9ZHR1JmlkPWR0dS1kNjZkMzM3MC0yNTZmLTQ4ODUtYTA5ZS0zMzZhOTI0ZjRiZWQmdGk9U2VkaW1lbnQlMjBtaWNyb2JpYWwlMjBmdWVsJTIwY2VsbHMlMjBmb3IlMjBsYWtlJTIwcmVzdG9yYXRpb24iLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vYmFja2VuZC5vcmJpdC5kdHUuZGsvd3MvZmlsZXMvMjQ4MDY5NTg5L1RoZXNpc19vbmxpbmVfdmVyc2lvbl9LYXJsX0hheHRoYXVzZW4ucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjEtMDEtMDEiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA4IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJSZXNlYXJjaCBQb3J0YWwgRGVubWFyayIsImltcGFjdEZhY3RvciI6IjAuMDQifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjo3Nn19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJUZWNobmljYWwgVW5pdmVyc2l0eSBvZiBEZW5tYXJrIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjowLCJjaXRlZEJ5Q291bnQiOjF9fSwicXVvdGUiOiJFeHRlcm5hbCBpbnB1dHMgaW5jcmVhc2UgUCBjb25jZW50cmF0aW9uIGluICB0aGUgd2F0ZXIsIGNhdXNpbmcgYWxnYWwgYmxvb21zIHRoYXQgc2hhZG93IHRoZSBsYWtlIGJvdHRvbSwgYW5kIGJlbnRoaWMgbWFjLXJvcGh5dGVzIHJlY2VkZS4gUHJlZGF0b3J5IGZpc2ggYXJlIHVuYWJsZSB0byBodW50LCBhbmQgdGhlaXIgZm9vZCBzb3VyY2UgaXMgIGRpc3J1cHRlZCBzdWNoIGFzIGZpc2ggZmVlZGluZyBvbiBiZW50aGljIGludmVydGVicmF0ZXMgYW5kIGJvdHRvbS1kd2VsbGluZyBhbmltYWxzIGFyZSBzdWZmb2NhdGVkIChWYW5kZXIgWmFuZGVuIGFuZCBWYWRlYm9uY29ldXIgMjAwMiksIHNvIHRoYXQgcGxhbmt0aXZvcm91cyBmaXNoIHNwZWNpZXMgZG9taW5hdGUuIFRoaXMgaW5jcmVhc2VzIHByZWRhdGlvbiBvbiB6b28gcGxhbmt0b24gIGFuZCByZWR1Y2VzIGdyYXppbmcgb24gYWxnYWUuIEFsZ2FlIHByb2xpZmVyYXRlLCBkaWUsIGFuZCBzaW5rIHRvIHRoZSBib3R0b20sICBjb25zdW1pbmcgb3h5Z2VuIHdoZW4gZGVncmFkZWQsIHdoaWNoIGxlYWRzIHRvIG94eWdlbiBkZWZpY2l0IGluIHRoZSBib3QtdG9tLCBraWxsaW5nIGZpc2ggKFd1cnRzYmF1Z2ggZXQgYWwuIDIwMTkpLiBGaWd1cmUgMjogVmljaW91cyBjeWNsZXMgb2YgZXV0cm9waGljYXRpb24uIExlZnQ6IEludGVybGlua2VkIHByb2Nlc3NlcyBzdXN0YWluaW5nIGV1dHJvcGhpLWNhdGlvbiBpbiBhIGxha2UuIEJsdWU6IHByb2Nlc3NlcyBpbiB3YXRlciwgZ3JlZW46IHByb2Nlc3NlcyBpbnZvbHZpbmcgbWljcm9hbGdhZSwgZ3JleTogc2VkLWltZW50IHJlbGF0ZWQgcHJvY2Vzc2VzLCB5ZWxsb3c6IHByb2Nlc3NlcyBpbnZvbHZpbmcgZmF1bmEuIFJpZ2h0LCBkYXNoZWQgbGluZTogVGhlIG9yZGVyIGluIHdoaWNoIGVsZWN0cm9uIGFjY2VwdG9ycyBhcmUgY29uc3VtZWQgaW4gdGhlIGFic2VuY2Ugb2YgTzIgc2hvd24gYXMgY29uY2VudHJhdGlvbiB2ZXJzdXMgIGRlcHRoIGluIHNlZGltZW50LCBhbmQgYXQgd2hpY2ggcG90ZW50aWFsIHRoaXMgb2NjdXJzIChCZXJuZXIgMTk4MSwgYnkgY291cnRlc3kgb2YgdGhlIHNvY2lldHkgIGZvciBzZWRpbWVudGFyeSBnZW9sb2d5KS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjE3LCJiYm94Ijp7InRvcCI6NzkuOTMzMTUxMjQ1MTE3MTksImxlZnQiOjE0LjI5NTUwMzYxNjMzMzAwOCwid2lkdGgiOjcxLjkzODkzNDMyNjE3MTg4LCJoZWlnaHQiOjIuMDUzNDc3Mjg3MjkyNDgwNX19LHsicGFnZU51bWJlciI6MTcsImJib3giOnsidG9wIjo4MS45NzEzNDM5OTQxNDA2MiwibGVmdCI6MTQuMjk1NTAzNjE2MzMzMDA4LCJ3aWR0aCI6NzEuMzMzMDAwMTgzMTA1NDcsImhlaWdodCI6Mi4wNTM0NzcyODcyOTI0ODA1fX0seyJwYWdlTnVtYmVyIjoxNywiYmJveCI6eyJ0b3AiOjg0LjAyMzc5NjA4MTU0Mjk3LCJsZWZ0IjoxNC4yOTU1MDM2MTYzMzMwMDgsIndpZHRoIjo3MS44MTIwMTkzNDgxNDQ1MywiaGVpZ2h0IjoyLjA1MzQ3NzI4NzI5MjQ4MDV9fSx7InBhZ2VOdW1iZXIiOjE3LCJiYm94Ijp7InRvcCI6ODYuMDYxOTk2NDU5OTYwOTQsImxlZnQiOjE0LjI5NTUwMzYxNjMzMzAwOCwid2lkdGgiOjcxLjk1NzM1OTMxMzk2NDg0LCJoZWlnaHQiOjIuMDUzNDc3Mjg3MjkyNDgwNX19LHsicGFnZU51bWJlciI6MTgsImJib3giOnsidG9wIjo5LjkyNjQ4NzkyMjY2ODQ1NywibGVmdCI6MTQuMjk1NTAzNjE2MzMzMDA4LCJ3aWR0aCI6NzEuOTc0MDYwMDU4NTkzNzUsImhlaWdodCI6MTAuMjM0Nzc3NDUwNTYxNTIzfX0seyJwYWdlTnVtYmVyIjoxOCwiYmJveCI6eyJ0b3AiOjQ0LjczNDA0MzEyMTMzNzg5LCJsZWZ0IjoxNC4yOTU1MDM2MTYzMzMwMDgsIndpZHRoIjo3MS44OTM0MzI2MTcxODc1LCJoZWlnaHQiOjEwLjQwMzI4OTc5NDkyMTg3NX19XX0sImNodW5rSWQiOiJkYmFlYTI3Mi00MGEzLTQzNmItYmY4Mi0yNmUyM2M3NGYyZTIiLCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI5MyJ9fV0%3D.

4. Agricultural Runoff: Fertilizers, Pesticides, and Monsoon Washout

India's agriculture, supporting over 1.4 billion people, relies heavily on intensive fertilizer application, making agricultural runoff a dominant contributor to eutrophication in lakes (Kumar & Mahajan, 2020; Nair & Nayak, 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hdnlhIFZpa3JhbWFuIiwiZmFtaWx5IjoiTmFpciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQcmF0ZWVwIEt1bWFyIiwiZmFtaWx5IjoiTmF5YWsifX1dLCJpZHMiOnsiaXNzbiI6WyIyMjk2LTc3NDUiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzODkvZm1hcnMuMjAyMy4xMDg3Mjk2Iiwib3BlbmFsZXgiOiJXNDM4MjcyMzEzMyJ9LCJ0aXRsZSI6IlVuY292ZXJpbmcgd2F0ZXIgcXVhbGl0eSBhbmQgZXZhbHVhdGluZyB2dWxuZXJhYmlsaXRpZXMgb2Ygc21hbGwtc2NhbGUgZmlzaGVyaWVzIGluIENoaWxpa2EgTGFnb29uLCBJbmRpYSIsImFic3RyYWN0IjoiU21hbGwtc2NhbGUgZmlzaGVyaWVzIChTU0ZzKSBwbGF5IGEgc3Ryb25nIHJvbGUgaW4gc3VzdGFpbmluZyBtaWxsaW9ucyBvZiBsaXZlbGlob29kcywgZm9vZCBzZWN1cml0eSwgbnV0cml0aW9uLCBhbmQgaW5jb21lIGdsb2JhbGx5IGJ1dCB0aGUgZmlzaGVycyBlbmdhZ2VkIGluIHRoaXMgc2VjdG9yIHNpbXVsdGFuZW91c2x5IGV4cGVyaWVuY2UgaGlnaCBsZXZlbHMgb2YgdnVsbmVyYWJpbGl0eSBhbmQgcHJvY2Vzc2VzIG9mIG1hcmdpbmFsaXNhdGlvbi4gU2V2ZXJhbCBmYWN0b3JzIGFyZSBhdHRyaWJ1dGVkIHRvIHRoZSBtdWx0aWRpbWVuc2lvbmFsIHZ1bG5lcmFiaWxpdGllcyB0aGUgc21hbGwtc2NhbGUgZmlzaGVycyBleHBlcmllbmNlLCBzcGFubmluZyBib3RoIG5hdHVyYWwgKGUuZy4sIG5hdHVyYWwgZGlzYXN0ZXJzLCBlY29zeXN0ZW0gY2hhbmdlKSBhbmQgYW50aHJvcG9nZW5pYyAoZS5nLiwgcG9saWN5IGNoYW5nZSwgaHlkcm9sb2dpY2FsIGludGVydmVudGlvbnMsIGFxdWFjdWx0dXJlKSBwcmVzc3VyZXMuIFdoaWxlIHRoZXJlIGlzIG11Y2ggbGl0ZXJhdHVyZSBvbiB2YXJpb3VzIG5hdHVyYWwgYW5kIGh1bWFuIGRyaXZlcnMgb2YgdnVsbmVyYWJpbGl0eSBpbiBzbWFsbC1zY2FsZSBmaXNoZXJ5IGNvbW11bml0aWVzLCBhbiBhYnNlbmNlIG9mIHJlc2VhcmNoIGNvbm5lY3RpbmcgdnVsbmVyYWJpbGl0eSB3aXRoIHdhdGVyIHF1YWxpdHkgaXMgZXZpZGVudC4gRmlzaGVyIGNvbW11bml0aWVzIG9mdGVuIHRhbGsgYWJvdXQgZmlzaCBpbiByZWxhdGlvbiB0byB0aGUgaGVhbHRoIG9mIHRoZWlyIGFxdWF0aWMgaGFiaXRhdHMgd2hlcmVpbiB3YXRlciBxdWFsaXR5IGlzIHNlZW4gYXMgYSBrZXkgcGFyYW1ldGVyLiBUaGUgbGluayBiZXR3ZWVuIGhlYWx0aHkgZmlzaCBhbmQgZ29vZCBxdWFsaXR5IHdhdGVyIGhhcyBzaWduaWZpY2FudCBpbXBsaWNhdGlvbnMgZm9yIHN0cm9uZyBhbmQgdmlhYmxlIGZpc2hpbmcgY29tbXVuaXRpZXMuIFRoaXMgcGFwZXIgZXhhbWluZXMgdGhlc2UgbGlua3MgZnVydGhlciBieSBmb2N1c2luZyBvbiB0aGUgbmF0dXJlIG9mIHZ1bG5lcmFiaWxpdGllcyBjYXVzZWQgYnkgd2F0ZXIgcXVhbGl0eSBjaGFuZ2VzIGluIHRoZSBzbWFsbC1zY2FsZSBmaXNoZXJ5IHN5c3RlbSBvZiBDaGlsaWthIExhZ29vbiBpbiBJbmRpYS4gV2UgdW5kZXJ0YWtlIGRldGFpbGVkIGFuYWx5c2lzIG9mIHRoZSBpbnZhc2l2ZSBzaHJpbXAgYXF1YWN1bHR1cmUgYWN0aXZpdGllcyBhbmQgaHlkcm9sb2dpY2FsIGludGVydmVudGlvbnMgZm9yIG9wZW5pbmcgb2YgYSBsYWdvb24gaW5sZXQgd2l0aCB0aGUgQmF5IG9mIEJlbmdhbCBhcyB0d28gZG9taW5hbnQgZHJpdmVycyBhZHZlcnNlbHkgaW1wYWN0aW5nIHdhdGVyIHF1YWxpdHkgYW5kIGluY3JlYXNpbmcgdnVsbmVyYWJpbGl0aWVzIG9mIHRoZSBlbnRpcmUgc21hbGwtc2NhbGUgZmlzaGVyaWVzIHNvY2lhbC1lY29sb2dpY2FsIHN5c3RlbS4gT3VyIGFuYWx5c2lzIHN1Z2dlc3RzIHRoYXQgdGhlcmUgYXJlIHN0cm9uZyBpbnRlcmNvbm5lY3Rpb25zIGJldHdlZW4gY2hhbmdlcyBpbiB3YXRlciBxdWFsaXR5IGFuZCB0aGUgbGV2ZWxzIG9mIHZ1bG5lcmFiaWxpdGllcyBpbiB0aGUgU1NGcyBvZiBDaGlsaWthIExhZ29vbi4gUG9sbHV0YW50cyBzdWNoIGFzIHBlc3RpY2lkZXMsIGFuZCBvcmdhbmljIGNvbXBvdW5kcyBhY2N1bXVsYXRlIGluIGZpc2ggdGlzc3VlcyBhbmQgYWZmZWN0IHRoZWlyIGdyb3d0aCwgcmVwcm9kdWN0aW9uLCBhbmQgb3ZlcmFsbCBoZWFsdGguIFRoaXMgbGVkIHRvIGRlY2xpbmVzIGluIGZpc2ggcG9wdWxhdGlvbnMsIG1ha2luZyBpdCBtb3JlIGRpZmZpY3VsdCBmb3IgZmlzaGVycyB0byBtYWtlIGEgbGl2aW5nLiBJbiBhZGRpdGlvbiB0byBkaXJlY3QgaW1wYWN0cyBvbiBmaXNoIHBvcHVsYXRpb25zLCBwb29yIHdhdGVyIHF1YWxpdHkgYWxzbyBoYXMgaW5kaXJlY3QgZWZmZWN0cyBvbiB0aGUgc29jaWFsIGFuZCBlY29ub21pYyB2dWxuZXJhYmlsaXR5IG9mIFNTRnMuIEZvciBleGFtcGxlLCBjb250YW1pbmF0aW9uIG9mIHdhdGVyIHNvdXJjZXMgbGVkIHRvIHRoZSByZWR1Y2VkIG51bWJlciBvZiBmaXNoIHNwZWNpZXMgcmVkdWNpbmcgdGhlIGFtb3VudCBvZiB0aW1lIGZpc2hlcnMgY2FuIHNwZW5kIG9uIGZpc2hpbmcgYWN0aXZpdGllcy4gVGhpcyBhbHNvIGFmZmVjdGVkIHRoZSBtYXJrZXRhYmlsaXR5IG9mIGZpc2ggcHJvZHVjdHMsIHJlZHVjZWQgaW5jb21lIGFuZCBpbmNyZWFzZWQgcG92ZXJ0eS4gVG8gZnVsbHkgdW5kZXJzdGFuZCB0aGUgaW50ZXJjb25uZWN0aW9ucyBiZXR3ZWVuIHdhdGVyIHF1YWxpdHkgYW5kIHZ1bG5lcmFiaWxpdHkgaW4gU1NGcyBpbiBDaGlsaWthIExhZ29vbiwgaXQgaXMgaW1wb3J0YW50IHRvIGNvbnNpZGVyIGJvdGggZW52aXJvbm1lbnRhbCBhbmQgc29jaWFsIGZhY3RvcnMsIGFzIHdlbGwgYXMgdGhlIGNvbXBsZXggZmVlZGJhY2sgbG9vcHMgYmV0d2VlbiB0aGVzZSBmYWN0b3JzLiBUaGUgc3R1ZHkgaGVscHMgaW4gYnJpZGdpbmcgYSBjcnVjaWFsIGdhcCBpbiBvdXIgdW5kZXJzdGFuZGluZyBvZiB0aGUgcm9sZSBvZiB3YXRlciBxdWFsaXR5IGluIHZ1bG5lcmFiaWxpdHkgYW5hbHlzaXMgd2l0aGluIHJlc291cmNlIGRlcGVuZGVudCBjb21tdW5pdGllcy4gV2UgY29uY2x1ZGUgd2l0aCBrZXkgaW5zaWdodHMgb24gcG9zc2libGUgY29waW5nIHJlc3BvbnNlcyBhbmQgYWRhcHRpdmUgY2FwYWNpdHkgbmVjZXNzYXJ5IGZvciB0aGUgc21hbGwtc2NhbGUgZmlzaGVyaWVzIGNvbW11bml0aWVzIHRvIHRyYW5zaXRpb24gdG93YXJkIHZpYWJpbGl0eS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMzM4OS9mbWFycy4yMDIzLjEwODcyOTYiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vd3d3LmZyb250aWVyc2luLm9yZy9hcnRpY2xlcy8xMC4zMzg5L2ZtYXJzLjIwMjMuMTA4NzI5Ni9wZGYiLCJpc3N1ZWREYXRlIjoiMjAyMy0wNi0yOSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZyb250aWVycyBpbiBNYXJpbmUgU2NpZW5jZSIsInZvbHVtZSI6MTAsImltcGFjdEZhY3RvciI6IjQuMjgifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRnJvbnRpZXJzIE1lZGlhIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo1MSwiY2l0ZWRCeUNvdW50Ijo1fX0sInF1b3RlIjoiV2F0ZXIgcXVhbGl0eSBvZiBDaGlsaWthIExhZ29vbiBoYXMgZGVncmFkZWQgZHVlIHRvIGFudGhyb3BvZ2VuaWMgaW50ZXJ2ZW50aW9ucyBhbmQgbmF0dXJhbCBmb3JjZXMgb2YgY2hhbmdlLiBWYXJpb3VzIGFjdGl2aXRpZXMgaW4gdGhlIENoaWxpa2EgTGFnb29uLCBzdWNoIGFzIGluZHVzdHJpYWwgd2FzdGV3YXRlciBkaXNwb3NhbCwgc2V3YWdlIGR1bXBpbmcsIGFxdWFjdWx0dXJlLCBoeWRyb2xvZ2ljYWwgaW50ZXJ2ZW50aW9ucywgYW5kIGN5Y2xvbmVzLCBhcmUgcHJvZHVjaW5nIHNhbGluaXR5IGNoYW5nZXMsIHNlZGltZW50IGRlcG9zaXRpb24sIG51dHJpZW50IGVucmljaG1lbnQsIGV1dHJvcGhpY2F0aW9uLCBhbmQgZGVhZCB6b25lcywgYXMgc3RhdGVkLiBGaXNoIHN1cnZpdmFsIGlzIGF0IHJpc2sgYXMgYSByZXN1bHQgb2YgdGhlIHNpZ25pZmljYW50IHJlZHVjdGlvbiBpbiBlbnZpcm9ubWVudGFsIGNpcmN1bXN0YW5jZXMsIHdoaWNoIGRpbWluaXNoZXMgZmlzaGVybWVu4oCZcyBpbmNvbWUgYW5kIHB1c2hlcyB0aGVtIG91dCBvZiBzb2NpZXR5LiBUaGVzZSBjYXVzZXMgYWZmZWN0aW5nIHRoZSB3YXRlciBxdWFsaXR5IG9mIHRoZSBMYWdvb24gZGVwaWN0IG1hbnkgZmFjZXMgb2YgdnVsbmVyYWJpbGl0eSBpbiBTU0YgY29tbXVuaXRpZXMuIEFzIHRoZSBDaGlsaWthIExhZ29vbiBlbnZpcm9ubWVudCBpcyBleHBlY3RlZCB0byB1bmRlcmdvIGluY3JlYXNpbmdseSBkcmFzdGljIGNoYW5nZXMgKHN1Y2ggYXMgY2F0Y2ggY29tcG9zaXRpb24sIGNhdGNoIGNhcGFjaXR5LCBiaW9kaXZlcnNpdHkgdmFyaWF0aW9ucywgd2F0ZXIgcXVhbGl0eSwgYW5kIGZpc2hlcnkgcmV2ZW51ZXMpLCBpbnRlcm5hdGlvbmFsIHBvbGljeSBoYXMgYmVjb21lIG1vcmUgaW50ZXJlc3RlZCBpbiBhZGFwdGF0aW9uIG1lYXN1cmVzLiBHaXZlbiBob3cgZGl2ZXJzZSBTU0YgZWNvc3lzdGVtcyBhcmUgZXhwZWN0ZWQgdG8gYmUgYmV0d2VlbiBjb250aW5lbnRzLCB0aGVyZSBpcyBubyDigJxvbmUgc2l6ZSBmaXRzIGFsbOKAnSBhcHByb2FjaC4gVGhlIHBhcGVyIG91dGxpbmVkIHNldmVyYWwgY29waW5nIGFuZCBhZGFwdGF0aW9uIG1lY2hhbmlzbXMgdG8gaW1wcm92ZSB3YXRlciBxdWFsaXR5IGFuZCBjb250aW51ZSBmaXNoaW5nIGluIGEgc3VzdGFpbmFibGUgd2F5LiBBZGFwdGF0aW9uIHN0cmF0ZWdpZXMgY2FuIG9jY2FzaW9uYWxseSBoYXZlIHVuZm9yZXNlZW4gY29uc2VxdWVuY2VzIGZvciBmaXNoZXJpZXMgYW5kIGZpc2hpbmcgY29tbXVuaXRpZXMuIEFueSB1bmV4cGVjdGVkIGNvbnNlcXVlbmNlcyBvZiBhZGFwdGF0aW9uIGNhbiBiZSBtaXRpZ2F0ZWQgdGhyb3VnaCBsb25nLXRlcm0gcGxhbm5pbmcgYW5kIGlkZW50aWZpY2F0aW9uIG9mIGZ1dHVyZSBvcHRpb25zIHRocm91Z2ggc2NlbmFyaW8gYW5hbHlzaXMuIFRoZSBmaW5kaW5ncyBvZiB0aGlzIHBhcGVyIGNhbiBiZSB1dGlsaXplZCB0byBkZXZlbG9wIHBvbGljaWVzIHRvIHByb3RlY3Qgd2F0ZXIgcXVhbGl0eSBhbmQgU1NGcyBpbiBhIHZhcmlldHkgb2YgY29hc3RhbCBsYWdvb24gc2V0dGluZ3MuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjIyLjI4NTAzNjA4NzAzNjEzMywibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ5MzA1MzQzNjI3OTMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjIzLjc3MzUxOTUxNTk5MTIxLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDgzMzY0MTA1MjI0NjEsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjI1LjI1NTU4NDcxNjc5Njg3NSwibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ4OTU1OTE3MzU4Mzk4NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjYuNzQ0MDY4MTQ1NzUxOTUzLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNTA2ODA1NDE5OTIxODc1LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyOC4yMjYxMzMzNDY1NTc2MTcsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40OTA0NjMyNTY4MzU5NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjkuNzE0NjE2Nzc1NTEyNjk1LCJsZWZ0Ijo4LjMzMTU5MjU1OTgxNDQ1Mywid2lkdGgiOjQwLjQ5NDA5ODY2MzMzMDA4LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozMS4xOTY2ODM4ODM2NjY5OTIsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40OTQzOTIzOTUwMTk1MywiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MzIuNjg1MTY1NDA1MjczNDQsImxlZnQiOjguMzMxNTk2Mzc0NTExNzE5LCJ3aWR0aCI6NDAuNDg3NDQ1ODMxMjk4ODMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM0LjE2NzIzMjUxMzQyNzczNCwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40OTc3NDU1MTM5MTYwMTYsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM1LjY1NTcxNTk0MjM4MjgxLCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ5ODYzODE1MzA3NjE3LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozNy4xMzc3ODMwNTA1MzcxMSwibGVmdCI6MTAuNjg1MTg2Mzg2MTA4Mzk4LCJ3aWR0aCI6MzguMTQxMTA1NjUxODU1NDcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM4LjYyNjI2NjQ3OTQ5MjE5LCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4NzE1MjA5OTYwOTM3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDAuMTA4MzMzNTg3NjQ2NDg0LCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4OTg1NjcxOTk3MDcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQxLjU5NjgxMzIwMTkwNDMsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDg4NjY2NTM0NDIzODMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQzLjA3ODg4MDMxMDA1ODU5NCwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40ODE4NjExMTQ1MDE5NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDQuNTY3MzYzNzM5MDEzNjcsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDgzMDU4OTI5NDQzMzYsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQ2LjA0OTQzMDg0NzE2Nzk3LCJsZWZ0Ijo4LjMzMzEwODkwMTk3NzUzOSwid2lkdGgiOjQwLjUwMDc0NzY4MDY2NDA2LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo0Ny41Mzc5MTQyNzYxMjMwNSwibGVmdCI6OC4zMzMxMDg5MDE5Nzc1MzksIndpZHRoIjo0MC40OTU2MDU0Njg3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDkuMDE5OTgxMzg0Mjc3MzQ0LCJsZWZ0Ijo4LjMzMzEwNjA0MDk1NDU5LCJ3aWR0aCI6NDAuNTA2ODE2ODY0MDEzNjcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUwLjUwODQ2NDgxMzIzMjQyLCJsZWZ0Ijo4LjMzMzEwNjA0MDk1NDU5LCJ3aWR0aCI6NDAuNDg4NTA2MzE3MTM4NjcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUxLjk5MDUyODEwNjY4OTQ1LCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDk3NzI2NDQwNDI5NjksImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUzLjQ3OTAxMTUzNTY0NDUzLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6MzMuMTU5NDQ2NzE2MzA4NTk0LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo1NC45NjEwNzg2NDM3OTg4MywibGVmdCI6MTAuNjgzNjcwMDQzOTQ1MzEyLCJ3aWR0aCI6MzguMTM2NTk2Njc5Njg3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NTYuNDQ5NTYyMDcyNzUzOTA2LCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDg3MTQ0NDcwMjE0ODQ0LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEzIn19LHsib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlBhd2FuIiwiZmFtaWx5IjoiS3VtYXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW1icmlzaCBLdW1hciIsImZhbWlseSI6Ik1haGFqYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyNTIzLTM5NjMiLCIyNTIzLTM5NzEiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMDcvczQyNDUyLTAyMC0zMDgyLTgiLCJtYWciOiIzMDM1OTYwMjI4Iiwib3BlbmFsZXgiOiJXMzAzNTk2MDIyOCJ9LCJ0aXRsZSI6IlRyb3BoaWMgc3RhdHVzIGFuZCBpdHMgcmVndWxhdGluZyBmYWN0b3IgZGV0ZXJtaW5hdGlvbiBhdCB0aGUgUmV3YWxzYXIgTGFrZSwgbm9ydGh3ZXN0IEhpbWFsYXlhIChIUCksIEluZGlhLCBiYXNlZCBvbiBzZWxlY3RlZCBwYXJhbWV0ZXJzIGFuZCBtdWx0aXZhcmlhdGUgc3RhdGlzdGljYWwgYW5hbHlzaXMiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAwNy9zNDI0NTItMDIwLTMwODItOCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9saW5rLnNwcmluZ2VyLmNvbS9jb250ZW50L3BkZi8xMC4xMDA3L3M0MjQ1Mi0wMjAtMzA4Mi04LnBkZiIsImlzc3VlZERhdGUiOiIyMDIwLTA2LTIyIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiU04gQXBwbGllZCBTY2llbmNlcyIsImlzc3VlIjo3LCJ2b2x1bWUiOjIsImltcGFjdEZhY3RvciI6IjcuMzUifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiU3ByaW5nZXIgTmF0dXJlIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo0MCwiY2l0ZWRCeUNvdW50IjoxOX19LCJxdW90ZSI6IkNoYW5naW5nIHF1YWxpdHkgaGFzIGNyZWF0ZWQgdGhlIHdhdGVyIHNjYXJjaXR5IGFuZCAgYSBodWdlIG1hc3Mgb2YgcG9wdWxhdGlvbiB0aHVzIGJlZW4gZGVwcml2ZWQgZnJvbSB1c2luZyAgdGhlIGZyZXNod2F0ZXIgZm9yIHZhcmlvdXMgcHVycG9zZXMgWzMsIDRdLiBIb3dldmVyLCB0aGUgIG9ibm94aW91cyBtYXRlcmlhbHMgdGhhdCBzdGF5IGluIHdhdGVyIGJvZGllcyBmb3IgYSBsb25nLXRpbWUgcGVyaW9kIGJyaW5nIGNoYW5nZXMgaW4gdGhlIHBoeXNpb2NoZW1pY2FsIGNoYXItYWN0ZXJpc3RpY3MgYXMgd2VsbCBhcyB0aGUgbmF0dXJhbCBzY2VuYXJpbyBvZiB0aGUgbGFrZXMgWzVdLiAgVGhlIEhpbWFsYXlhbiBsYWtlcyBhcmUgdW5kZXIgc3RyZXNzIGR1ZSB0byBtaXNtYW5hZ2VkIGh1bWFuIGFjdGl2aXRpZXMsIHdoaWNoIGluY3JlYXNlIG51dHJpZW50cyBhbmQgcG9sbHV0aW9uIGluIHRlcm1zIG9mIGhlYXZ5IG1ldGFscyB0b3hpY2l0eSBhbmQgYmFjdGVyaWFsIGNvbnRhbWluYXRpb24gIFs24oCTOF0uIFRoZSBtYW5hZ2VtZW50IGFuZCBwcmVzZXJ2YXRpb24gb2YgdGhlIGxha2VzIGF0ICBwcmVzZW50IGhhdmUgYmVjb21lIGEgZ3JlYXQgY2hhbGxlbmdlIGZvciBlbnZpcm9ubWVudGFsLWlzdHMgYW5kIG1hbmFnZXJzIFs1LCA5XS4gVGhlIGV1dHJvcGhpY2F0aW9uIGlzIGFsc28gYSBtYWluIHByb2JsZW0gb2YgdGhlICBmcmVzaHdhdGVyIGxha2VzLCBhbmQgaXQgaXMgc3BlY2lmaWVkIGZvciB0aGUgbGFrZXMgaGF2aW5nICBudXRyaWVudCBlbnJpY2htZW50LiBJdCBjYXVzZXMgZXhjZXNzaXZlIGdyb3d0aCBvZiBhbGdhZSAgYW5kIGlzIGNvbnNpZGVyZWQgdG8gYmUgcmVzcG9uc2libGUgZm9yIGRlZ3JhZGluZyB0aGUgIHF1YWxpdGF0aXZlIG5hdHVyZSBvZiB0aGUgbGFrZXMgWzUsIDEwXS4gVGhlIGV1dHJvcGhpY2F0aW9uICBpcyBmb2xsb3dlZCBieSB0aGUgYWxnYWwgYmxvb20sIHdoaWNoIGdlbmVyYXRlcyB0b3hpbiwgIHdvcnNlbnMgd2F0ZXIgcXVhbGl0eSwgZGVwbGV0ZXMgZGlzc29sdmVkIG94eWdlbiAoRE8pICogUGF3YW4gS3VtYXIsIHBhd2FuLmV2c0BnbWFpbC5jb20gfCAxRGVwYXJ0bWVudCBvZiBFbnZpcm9ubWVudGFsIFNjaWVuY2VzLCBDZW50cmFsIFVuaXZlcnNpdHkgb2YgSGltYWNoYWwgUHJhZGVzaCwgRGhhcmFtc2hhbGEgMTc2MjE1LCBJbmRpYS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2OC41Nzg0NjA2OTMzNTkzOCwibGVmdCI6OC41NzE0MTg3NjIyMDcwMzEsIndpZHRoIjo0MC4zMjc1MTQ2NDg0Mzc1LCJoZWlnaHQiOjEuNTE5ODU1MjYwODQ4OTk5fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NzAuMTU4OTk2NTgyMDMxMjUsImxlZnQiOjguNTY5NzM4Mzg4MDYxNTIzLCJ3aWR0aCI6NDAuMzU0NzI0ODg0MDMzMiwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjcxLjczOTU0NzcyOTQ5MjE5LCJsZWZ0Ijo4LjU2OTczODM4ODA2MTUyMywid2lkdGgiOjQwLjM0OTY5MzI5ODMzOTg0NCwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjczLjMyMDA5MTI0NzU1ODYsImxlZnQiOjguNTY4MDU5OTIxMjY0NjQ4LCJ3aWR0aCI6NDAuMDIwMDg4MTk1ODAwNzgsImhlaWdodCI6MS41MTk4NTUyNjA4NDg5OTl9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo3NC45MDA2NDIzOTUwMTk1MywibGVmdCI6OC41NjgwNTk5MjEyNjQ2NDgsIndpZHRoIjozOS45OTk5Mjc1MjA3NTE5NSwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjc2LjQ4MTE4NTkxMzA4NTk0LCJsZWZ0Ijo4LjU2ODA1NzA2MDI0MTcsIndpZHRoIjo0MC4zNTcwNzQ3Mzc1NDg4MywiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjc4LjA2MTcyOTQzMTE1MjM0LCJsZWZ0Ijo4LjU2OTczNjQ4MDcxMjg5LCJ3aWR0aCI6NDAuMzUzNzEzOTg5MjU3ODEsImhlaWdodCI6MS41MTk4NTUyNjA4NDg5OTl9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2MC42NzU3Mjc4NDQyMzgyOCwibGVmdCI6NTEuNDI3MTY1OTg1MTA3NDIsIndpZHRoIjo0MC4zNzg1NjY3NDE5NDMzNiwiaGVpZ2h0IjoxOC45MDU4NTUxNzg4MzMwMDh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo4MS45MjczMzc2NDY0ODQzOCwibGVmdCI6OC40MDE0OTc4NDA4ODEzNDgsIndpZHRoIjo3My45NTQ2OTY2NTUyNzM0NCwiaGVpZ2h0IjoyLjY3ODc2MjY3NDMzMTY2NX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEifX1d. The contribution of agricultural runoff and monsoonal nutrient washout to HAB formation is illustrated in Figure 3. With annual fertilizer consumption exceeding 60 million tons, primarily nitrogenous (urea) and phosphatic compounds, farmlands surrounding lakes become hotspots for nutrient leaching (Haq et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjQsInN1YnR5cGUiOnt9LCJjb250cmlidXRvcnMiOlt7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJadWxmcWFydWwiLCJmYW1pbHkiOiJIYXEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU25vYmVyIiwiZmFtaWx5IjoiSXJzaGFkIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkF6bWF0IEFsYW0iLCJmYW1pbHkiOiJLaGFuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNoZWlraCBGLiIsImZhbWlseSI6IkFobWFkIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNob3drZWVuIE11emFtaWwiLCJmYW1pbHkiOiJCYXNoaXIifX1dLCJpZHMiOnsiaXNzbiI6WyIyNjMyLTA1MTciXSwiaXNibiI6W10sImRvaSI6IjEwLjU3NzIvaW50ZWNob3Blbi4xMDA2NTc4Iiwib3BlbmFsZXgiOiJXNDQwNDUyOTIwMyJ9LCJ0aXRsZSI6IkFkdmFuY2VzIGluIE1hbmFnaW5nIE5pdHJvZ2VuIGFuZCBQaG9zcGhvcnVzIEVtaXNzaW9ucyBpbiBSdW1pbmFudHM6IEEgSG9saXN0aWMgQXBwcm9hY2giLCJhYnN0cmFjdCI6IuKAnEJhbGFuY2luZyBOdXRyaXRpb24gYW5kIEVudmlyb25tZW50YWwgU3VzdGFpbmFiaWxpdHk6IEFkdmFuY2VzIGluIE1hbmFnaW5nIE5pdHJvZ2VuIGFuZCBQaG9zcGhvcnVzIEVtaXNzaW9ucyBpbiBSdW1pbmFudHPigJ0gZXhwbG9yZXMgdGhlIGludHJpY2F0ZSByZWxhdGlvbnNoaXAgYmV0d2VlbiBydW1pbmFudCBudXRyaXRpb24gYW5kIGVudmlyb25tZW50YWwgc3Rld2FyZHNoaXAuIEl0IGRlbHZlcyBpbnRvIHRoZSBkeW5hbWljcyBvZiBuaXRyb2dlbiBhbmQgcGhvc3Bob3J1cyBtZXRhYm9saXNtIGluIHJ1bWluYW50cywgYWRkcmVzc2luZyB0aGUgZW52aXJvbm1lbnRhbCByZXBlcmN1c3Npb25zIG9mIHRoZWlyIGVtaXNzaW9ucy4gVGhlc2UgZW1pc3Npb25zLCBwcmltYXJpbHkgaW4gdGhlIGZvcm0gb2YgYW1tb25pYSBhbmQgcGhvc3BoYXRlcywgcG9zZSBzaWduaWZpY2FudCBlbnZpcm9ubWVudGFsIGNoYWxsZW5nZXMsIGNvbnRyaWJ1dGluZyB0byBhaXIgYW5kIHdhdGVyIHBvbGx1dGlvbiwgc29pbCBkZWdyYWRhdGlvbiwgYW5kIGNsaW1hdGUgY2hhbmdlLiBUaGUgY2hhcHRlciBoaWdobGlnaHRzIGlubm92YXRpdmUgc3RyYXRlZ2llcyBhbmQgdGVjaG5vbG9naWNhbCBicmVha3Rocm91Z2hzIGFpbWVkIGF0IG1pdGlnYXRpbmcgbml0cm9nZW4gYW5kIHBob3NwaG9ydXMgZXhjcmV0aW9uLCBjcnVjaWFsIGZvciByZWR1Y2luZyBlbnZpcm9ubWVudGFsIHBvbGx1dGlvbi4gSXQgb2ZmZXJzIGluc2lnaHRzIGludG8gZGlldGFyeSBhZGp1c3RtZW50cywgZmVlZCBhZGRpdGl2ZXMsIGFuZCBiZXN0IHByYWN0aWNlcyBmb3Igc3VzdGFpbmFibGUgcnVtaW5hbnQgcHJvZHVjdGlvbi4gS2V5IGFyZWFzIG9mIGZvY3VzIGluY2x1ZGUgZGlldGFyeSBhZGp1c3RtZW50cyB0YWlsb3JlZCB0byBvcHRpbWl6ZSBudXRyaWVudCB1dGlsaXphdGlvbiBhbmQgcmVkdWNlIHdhc3RlLCB0aGUgdXRpbGl6YXRpb24gb2YgZmVlZCBhZGRpdGl2ZXMgdG8gZW5oYW5jZSBudXRyaWVudCBhYnNvcnB0aW9uIGFuZCBtaW5pbWl6ZSBleGNyZXRpb24sIGFuZCB0aGUgaW1wbGVtZW50YXRpb24gb2YgYmVzdCBtYW5hZ2VtZW50IHByYWN0aWNlcyB0byBwcm9tb3RlIHN1c3RhaW5hYmxlIHJ1bWluYW50IHByb2R1Y3Rpb24uIEJ5IGludGVncmF0aW5nIHNjaWVudGlmaWMgZmluZGluZ3Mgd2l0aCBwcmFjdGljYWwgYXBwbGljYXRpb25zLCB0aGlzIHJldmlldyB1bmRlcnNjb3JlcyB0aGUgaW1wb3J0YW5jZSBvZiBhY2hpZXZpbmcgaGFybW9ueSBiZXR3ZWVuIG51dHJpdGlvbiBvcHRpbWl6YXRpb24gYW5kIGVudmlyb25tZW50YWwgcmVzcG9uc2liaWxpdHkgaW4gbW9kZXJuIGFncmljdWx0dXJlLiBUaHJvdWdoIHRoaXMgY29tcHJlaGVuc2l2ZSBhbmFseXNpcywgdGhlIGNoYXB0ZXIgYWltcyBmb3IgYmFsYW5jaW5nIHRoZSBkdWFsIGdvYWxzIG9mIG9wdGltaXppbmcgcnVtaW5hbnQgbnV0cml0aW9uIGFuZCBtaW5pbWl6aW5nIGVudmlyb25tZW50YWwgaW1wYWN0LCB0aGVyZWJ5IGNvbnRyaWJ1dGluZyB0byBtb3JlIHN1c3RhaW5hYmxlIGFuZCByZXNpbGllbnQgYWdyaWN1bHR1cmFsIHN5c3RlbXMuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjU3NzIvaW50ZWNob3Blbi4xMDA2NTc4IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5pbnRlY2hvcGVuLmNvbS9jaXRhdGlvbi1wZGYtdXJsLzExODc4MTQiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMS0xNSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IlZldGVyaW5hcnkgbWVkaWNpbmUgYW5kIHNjaWVuY2UiLCJpbXBhY3RGYWN0b3IiOiIwLjk5In0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkludGVjaE9wZW4ifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjUyLCJjaXRlZEJ5Q291bnQiOjF9fSwicXVvdGUiOiJXYXRlciBRdWFsaXR5IENoYW5nZXMg4oCiIENsYXJpdHkgYW5kIEFsZ2FsIEJsb29tczogV2F0ZXIgY2xhcml0eSBtYXkgZGVjcmVhc2Ugc2xpZ2h0bHkgZHVlIHRvIHRoZSBpbml0aWFsIGFsZ2FsIGJsb29tcy4gSG93ZXZlciwgdGhpcyBwaGFzZSBpcyBvZnRlbiBzdGlsbCBjaGFyYWN0ZXJpemVkIGJ5IHJlbGF0aXZlbHkgZ29vZCB3YXRlciBxdWFsaXR5LiDigKIgQmlvZGl2ZXJzaXR5OiBUaGUgZWNvc3lzdGVtIG1pZ2h0IHN0aWxsIHN1cHBvcnQgYSBkaXZlcnNlIHJhbmdlIG9mIGFxdWF0aWMgb3JnYW5pc21zLCBpbmNsdWRpbmcgZmlzaCBhbmQgaW52ZXJ0ZWJyYXRlcy4gMy4xLjQuMiBQaGFzZSAyOiBNZXNvdHJvcGhpYyB0byBldXRyb3BoaWMgdHJhbnNpdGlvbiAxLiBOdXRyaWVudCBJbWJhbGFuY2Ug4oCiIFBob3NwaG9ydXMgdnMuIE5pdHJvZ2VuOiBBcyBhZ3JpY3VsdHVyYWwgcHJhY3RpY2VzIGludGVuc2lmeSBhbmQgdXJiYW5pemF0aW9uIGluY3JlYXNlcywgcGhvc3Bob3J1cyBpbnB1dHMgKGUuZy4sIGZyb20gZmVydGlsaXplcnMgYW5kIGRldGVyZ2VudHMpIG9mdGVuIGV4Y2VlZCB0aG9zZSBvZiBuaXRyb2dlbi4gVGhpcyBpbWJhbGFuY2UgY2FuIGRyaXZlIHNpZ25pZmljYW50IGNoYW5nZXMgaW4gd2F0ZXIgY2hlbWlzdHJ5IGFuZCBlY29zeXN0ZW0gZHluYW1pY3MuIDIuIEFsZ2FsIEJsb29tIEludGVuc2lmaWNhdGlvbiDigKIgQ3lhbm9iYWN0ZXJpYSBEb21pbmFuY2U6IEV4Y2VzcyBwaG9zcGhvcnVzIGNhbiBsZWFkIHRvIHRoZSBkb21pbmFuY2Ugb2YgY3lhbm9iYWN0ZXJpYSAoYmx1ZS1ncmVlbiBhbGdhZSksIHdoaWNoIGNhbiBvdXRjb21wZXRlIG90aGVyIGFsZ2FlIGFuZCBhcXVhdGljIHBsYW50cy4gVGhlc2UgYmxvb21zIGNhbiBiZSB0b3hpYyBhbmQgaGFybWZ1bCB0byBhcXVhdGljIGxpZmUuIOKAoiBIeXBveGlhOiBUaGUgZGVjb21wb3NpdGlvbiBvZiBsYXJnZSBhbGdhbCBibG9vbXMgY29uc3VtZXMgb3h5Z2VuLCBsZWFkaW5nIHRvIGh5cG94aWMgKGxvdyBveHlnZW4pIG9yIGFub3hpYyAobm8gb3h5Z2VuKSBjb25kaXRpb25zLCBwYXJ0aWN1bGFybHkgaW4gZGVlcGVyIHdhdGVycy4gMy4gV2F0ZXIgUXVhbGl0eSBEZXRlcmlvcmF0aW9uIOKAoiBEZWNyZWFzZWQgQ2xhcml0eTogV2F0ZXIgY2xhcml0eSBzaWduaWZpY2FudGx5IGRlY3JlYXNlcyBkdWUgdG8gZGVuc2UgYWxnYWwgZ3Jvd3RoLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo2Ny4yNjEwMDE1ODY5MTQwNiwibGVmdCI6MTUuNTgzNDU4OTAwNDUxNjYsIndpZHRoIjoyMC40NjA2MTg5NzI3NzgzMiwiaGVpZ2h0IjoxLjkwNTE0ODYyNTM3Mzg0MDN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NzAuNTEzMDMxMDA1ODU5MzgsImxlZnQiOjE3LjM5ODIxNjI0NzU1ODU5NCwid2lkdGgiOjY2LjQwMzU0OTE5NDMzNTk0LCJoZWlnaHQiOjEuOTA1MTU3MDg5MjMzMzk4NH19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo3Mi4xMzkwNDU3MTUzMzIwMywibGVmdCI6MTkuMDg4MzU2MDE4MDY2NDA2LCJ3aWR0aCI6NjUuNzg4NTIwODEyOTg4MjgsImhlaWdodCI6My41Mjg0NjA1MDI2MjQ1MTE3fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjc3LjAxNzA5NzQ3MzE0NDUzLCJsZWZ0IjoxNy4zOTgyMTYyNDc1NTg1OTQsIndpZHRoIjo2Mi4wOTQ1NzAxNTk5MTIxMSwiaGVpZ2h0IjoxLjkwNTE1NzA4OTIzMzM5ODR9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NzguNjQzMTEyMTgyNjE3MTksImxlZnQiOjE5LjA4ODM1NjAxODA2NjQwNiwid2lkdGgiOjM2LjcyNTMxMTI3OTI5Njg3NSwiaGVpZ2h0IjoxLjkwMjQ0NDI0MzQzMTA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6ODEuODk1MTMzOTcyMTY3OTcsImxlZnQiOjEzLjI0Njg1OTU1MDQ3NjA3NCwid2lkdGgiOjQwLjM5MTM5NTU2ODg0NzY1NiwiaGVpZ2h0IjoxLjkwNTE1NzA4OTIzMzM5ODR9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6ODUuMTQ3MTYzMzkxMTEzMjgsImxlZnQiOjE1LjU4MzQ1ODkwMDQ1MTY2LCJ3aWR0aCI6MTcuNjAyMTc4NTczNjA4NCwiaGVpZ2h0IjoxLjkwNTE1NzA4OTIzMzM5ODR9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6ODguMzk5MTkyODEwMDU4NiwibGVmdCI6MTcuMzk4MjE2MjQ3NTU4NTk0LCJ3aWR0aCI6NjUuMDA3MzQ3MTA2OTMzNiwiaGVpZ2h0IjoxLjkwNTE1NzA4OTIzMzM5ODR9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6OTAuMDI1MjA3NTE5NTMxMjUsImxlZnQiOjE5LjA4ODM1NjAxODA2NjQwNiwid2lkdGgiOjYxLjA1MTA1MjA5MzUwNTg2LCJoZWlnaHQiOjEuOTAyNDQ0MjQzNDMxMDkxM319LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjoxMC4zNTA0MTcxMzcxNDU5OTYsImxlZnQiOjIwLjg0MDgwNTA1MzcxMDkzOCwid2lkdGgiOjY2LjI3NzA1MzgzMzAwNzgxLCJoZWlnaHQiOjMuNTI4NDU1MjU3NDE1NzcxNX19LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjoxNS40OTk0Njc4NDk3MzE0NDUsImxlZnQiOjE3LjMzNTkwNjk4MjQyMTg3NSwid2lkdGgiOjIzLjUxOTYxNTE3MzMzOTg0NCwiaGVpZ2h0IjoxLjkwNTE0ODYyNTM3Mzg0MDN9fSx7InBhZ2VOdW1iZXIiOjEzLCJiYm94Ijp7InRvcCI6MTguNzUxNTAxMDgzMzc0MDIzLCJsZWZ0IjoxOS4xNTA2NjUyODMyMDMxMjUsIndpZHRoIjo2NC40Mzg3NjY0Nzk0OTIxOSwiaGVpZ2h0IjoxLjkwNTE0ODYyNTM3Mzg0MDN9fSx7InBhZ2VOdW1iZXIiOjEzLCJiYm94Ijp7InRvcCI6MjAuMzc3NTE3NzAwMTk1MzEyLCJsZWZ0IjoyMC44NDA4MDUwNTM3MTA5MzgsIndpZHRoIjo2MS4wNzQ0MjQ3NDM2NTIzNDQsImhlaWdodCI6My41Mjg0NTQ1NDIxNjAwMzR9fSx7InBhZ2VOdW1iZXIiOjEzLCJiYm94Ijp7InRvcCI6MjUuMjU1NTY3NTUwNjU5MTgsImxlZnQiOjE5LjE1MDY2NTI4MzIwMzEyNSwid2lkdGgiOjY4LjAyMzU1OTU3MDMxMjUsImhlaWdodCI6MS45MDUxNDg2MjUzNzM4NDAzfX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjI2Ljg4MTU4MjI2MDEzMTgzNiwibGVmdCI6MjAuODQwODA1MDUzNzEwOTM4LCJ3aWR0aCI6NjYuMDc2MjkzOTQ1MzEyNSwiaGVpZ2h0IjozLjUyODQ3MDk5MzA0MTk5Mn19LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjozMi4wMzA2NTEwOTI1MjkzLCJsZWZ0IjoxNy4zMzU5MDY5ODI0MjE4NzUsIndpZHRoIjoyNC42Nzc5NzY2MDgyNzYzNjcsImhlaWdodCI6MS45MDUxNDg2MjUzNzM4NDAzfX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjM1LjI4MjY4NDMyNjE3MTg3NSwibGVmdCI6MTkuMTUwNjY1MjgzMjAzMTI1LCJ3aWR0aCI6NjIuNDE5Nzg4MzYwNTk1NywiaGVpZ2h0IjoxLjkwNTE0ODYyNTM3Mzg0MDN9fSx7InBhZ2VOdW1iZXIiOjEzLCJiYm94Ijp7InRvcCI6MzYuOTA4Njk5MDM1NjQ0NTMsImxlZnQiOjIwLjg0MDgwNTA1MzcxMDkzOCwid2lkdGgiOjYuNjMzOTkzMTQ4ODAzNzExLCJoZWlnaHQiOjEuOTAyNDQwMTkwMzE1MjQ2Nn19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEzIn19XQ%3D%3D. During the monsoon season, intense rainfall often >1000 mm in peninsular regions washes these chemicals into watersheds via surface runoff, accounting for 50-70% of phosphorus and 30-50% of nitrogen inputs to inland waters (Sitote & Gebremedhine, 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllhYnNyYSBNZWxhayIsImZhbWlseSI6IlNpdG90ZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNaWNoYWVsIEdpcmltYXkiLCJmYW1pbHkiOiJHZWJyZW1lZGhpbmUifX1dLCJpZHMiOnsiaXNzbiI6W10sImlzYm4iOltdLCJkb2kiOiIxMC4yMDk0NC9wcmVwcmludHMyMDI0MDkuMDcwNC52MSIsIm9wZW5hbGV4IjoiVzQ0MDI0NjU0MTQiLCJleGEiOiJodHRwczovL3d3dy5wcmVwcmludHMub3JnL21hbnVzY3JpcHQvMjAyNDA5LjA3MDQifSwidGl0bGUiOiJDb21wcmVoZW5zaXZlIFJldmlldyBvZiBFdXRyb3BoaWNhdGlvbiBpbiBGcmVzaHdhdGVyIEVjb3N5c3RlbXM6IENhdXNlcywgRWZmZWN0cywgQXNzZXNzbWVudCwgYW5kIE1hbmFnZW1lbnQgU3RyYXRlZ2llcyIsImFic3RyYWN0IjoiVGhpcyByZXZpZXcgZXhhbWluZXMgZXV0cm9waGljYXRpb24gaW4gZnJlc2h3YXRlciBlY29zeXN0ZW1zLCBhIHdpZGVzcHJlYWQgZW52aXJvbm1lbnRhbCBpc3N1ZSBzdGVtbWluZyBmcm9tIGV4Y2Vzc2l2ZSBudXRyaWVudCBpbnB1dCwgbGFyZ2VseSBkdWUgdG8gaHVtYW4gYWN0aXZpdGllcy4gVGhlIHN0dWR5IGV4cGxvcmVzIHRoZSBvcmlnaW5zLCBjb25zZXF1ZW5jZXMsIGV2YWx1YXRpb24gdGVjaG5pcXVlcywgYW5kIGNvbnRyb2wgbWV0aG9kcyBmb3IgZXV0cm9waGljYXRpb24uIFByaW1hcnkgY2F1c2VzIGluY2x1ZGUgYWdyaWN1bHR1cmFsIGRpc2NoYXJnZSwgdXJiYW4gZGV2ZWxvcG1lbnQsIGFuZCBjbGltYXRlIHNoaWZ0cy4gVGhlIGVmZmVjdHMgYXJlIGZhci1yZWFjaGluZywgaW1wYWN0aW5nIGJpb2RpdmVyc2l0eSwgd2F0ZXIgcXVhbGl0eSwgYW5kIHNvY2lvLWVjb25vbWljIGZhY3RvcnMuIFRoZSBhc3Nlc3NtZW50IHV0aWxpemVzIGEgY29tYmluYXRpb24gb2YgY2hlbWljYWwsIHBoeXNpY2FsLCBhbmQgYmlvbG9naWNhbCBtYXJrZXJzLCB3aXRoIGN1dHRpbmctZWRnZSB0ZWNobm9sb2dpZXMgbGlrZSByZW1vdGUgc2Vuc2luZyBpbXByb3ZpbmcgbW9uaXRvcmluZyBlZmZvcnRzLiBNaXRpZ2F0aW9uIHN0cmF0ZWdpZXMgYXJlIGRpdmVyc2UsIGluY29ycG9yYXRpbmcgcHJldmVudGF0aXZlIGFjdGlvbnMgKHN1Y2ggYXMgbnV0cmllbnQgbGltaXRhdGlvbiBhbmQgbGFuZCBtYW5hZ2VtZW50KSwgY29ycmVjdGl2ZSBtZWFzdXJlcyAoaW5jbHVkaW5nIGJpby1tYW5pcHVsYXRpb24gYW5kIGNoZW1pY2FsIGFwcGxpY2F0aW9ucyksIGFuZCByZWd1bGF0b3J5IGluaXRpYXRpdmVzIChsaWtlIHdhdGVyIHF1YWxpdHkgZ3VpZGVsaW5lcyBhbmQgbnV0cmllbnQgZXhjaGFuZ2Ugc2NoZW1lcykuIFdoaWxlIGNhc2Ugc3R1ZGllcyBkZW1vbnN0cmF0ZSBzdWNjZXNzZnVsIGludGVydmVudGlvbnMsIHRoZXkgYWxzbyByZXZlYWwgb25nb2luZyBjaGFsbGVuZ2VzLCBwYXJ0aWN1bGFybHkgZWNvc3lzdGVtIHVucHJlZGljdGFiaWxpdHkgYW5kIGNsaW1hdGUgY2hhbmdlIGluZmx1ZW5jZXMuIFRoZSByZXNlYXJjaCBoaWdobGlnaHRzIHRoZSBwcm9taXNlIG9mIGVtZXJnaW5nIHRlY2hub2xvZ2llcywgbm90YWJseSBtYWNoaW5lIGxlYXJuaW5nLCBpbiBlbmhhbmNpbmcgbWFuYWdlbWVudCBwcmFjdGljZXMuIEtub3dsZWRnZSBnYXBzIHBlcnNpc3QsIGVzcGVjaWFsbHkgcmVnYXJkaW5nIGV1dHJvcGhpY2F0aW9uLWJpb2RpdmVyc2l0eSBkeW5hbWljcyBpbiB0cm9waWNhbCBlbnZpcm9ubWVudHMgYW5kIHRoZSBzb2Npby1lY29ub21pYyByYW1pZmljYXRpb25zIG9mIGludGVydmVudGlvbnMuIFRoZSBzdHVkeSBlbXBoYXNpemVzIHRoZSBpbXBvcnRhbmNlIG9mIGZsZXhpYmxlLCBjb21wcmVoZW5zaXZlIGFwcHJvYWNoZXMgdGhhdCBibGVuZCBzY2llbnRpZmljIGluc2lnaHRzIHdpdGggcHJhY3RpY2FsIHNvbHV0aW9ucyBhbmQgc3Ryb25nIHBvbGljeSBzdHJ1Y3R1cmVzIHRvIHRhY2tsZSB0aGlzIGludHJpY2F0ZSBlbnZpcm9ubWVudGFsIGNoYWxsZW5nZSBpbiBhbiBlcmEgb2YgY2xpbWF0ZSBjaGFuZ2UuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjIwOTQ0L3ByZXByaW50czIwMjQwOS4wNzA0LnYxIiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5wcmVwcmludHMub3JnL2Zyb250ZW5kL21hbnVzY3JpcHQvZTA4ZTViNTY1MzNlMmFhZDc0N2ZkOTQzNWU2N2NhNzMvZG93bmxvYWRfcHViIiwiaXNzdWVkRGF0ZSI6IjIwMjQtMDktMTAiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTEyIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJQcmVwcmludHMub3JnIiwiaW1wYWN0RmFjdG9yIjoiMS4xNSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MCwiY2l0ZWRCeUNvdW50Ijo1fX0sInF1b3RlIjoiVGhlIHN0dWR5IGVtcGhhc2l6ZXMgdGhlIGltcG9ydGFuY2Ugb2YgZmxleGlibGUsIGNvbXByZWhlbnNpdmUgYXBwcm9hY2hlcyB0aGF0IGJsZW5kIHNjaWVudGlmaWMgaW5zaWdodHMgd2l0aCBwcmFjdGljYWwgc29sdXRpb25zIGFuZCBzdHJvbmcgcG9saWN5IHN0cnVjdHVyZXMgdG8gdGFja2xlIHRoaXMgaW50cmljYXRlIGVudmlyb25tZW50YWwgY2hhbGxlbmdlIGluIGFuIGVyYSBvZiBjbGltYXRlIGNoYW5nZS4gRXV0cm9waGljYXRpb24sIGNhdXNlZCBieSBleGNlc3NpdmUgbnV0cmllbnQgZW5yaWNobWVudCBpbiB3YXRlciBib2RpZXMsIGlzIGEgc2lnbmlmaWNhbnQgZW52aXJvbm1lbnRhbCBjaGFsbGVuZ2UgZm9yIGZyZXNod2F0ZXIgZWNvc3lzdGVtcyBvbiBhIGdsb2JhbCBzY2FsZS4gUHJpbWFyaWx5IGRyaXZlbiBieSBuaXRyb2dlbiBhbmQgcGhvc3Bob3J1cyBpbnB1dHMsIHRoaXMgcHJvY2VzcyByZXN1bHRzIGluIHRoZSBvdmVycHJvZHVjdGlvbiBvZiBhbGdhZSBhbmQgYXF1YXRpYyBwbGFudHMsIGxlYWRpbmcgdG8gYSBjYXNjYWRlIG9mIGVjb2xvZ2ljYWwgZGlzdHVyYmFuY2VzLCBpbmNsdWRpbmcgb3h5Z2VuIGRlcGxldGlvbiAoaHlwb3hpYSksIGxvc3Mgb2YgYmlvZGl2ZXJzaXR5LCBhbmQgc2V2ZXJlIHdhdGVyIHF1YWxpdHkgZGVncmFkYXRpb24gKEtha2FkZSBldCBhbC4sIDIwMjE7IE1lZXJob2ZmIGV0IGFsLiwgMjAyMjsgV3VydHNiYXVnaCBldCBhbC4sIDIwMTkpLiJ9XQ%3D%3D. This seasonal pulse elevates total phosphorus to 200-500 μg/L and total nitrogen to 2-5 mg/L in receiving lakes, surpassing eutrophic thresholds and triggering phytoplankton explosions (Jilbert et al., 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRvbSIsImZhbWlseSI6IkppbGJlcnQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUmFvdWzigJBNYXJpZSIsImZhbWlseSI6IkNvdXR1cmUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQnJpYW4iLCJmYW1pbHkiOiJIdXNlciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLYWxldmkiLCJmYW1pbHkiOiJTYWxvbmVuIn19XSwiaWRzIjp7Imlzc24iOlsiMDAxOC04MTU4IiwiMTU3My01MTE3Il0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDA3L3MxMDc1MC0wMjAtMDQ0NTcteCIsIm1hZyI6IjMxMDA0NTkwNTAiLCJvcGVuYWxleCI6IlczMTAwNDU5MDUwIn0sInRpdGxlIjoiUHJlZmFjZTogUmVzdG9yYXRpb24gb2YgZXV0cm9waGljIGxha2VzOiBjdXJyZW50IHByYWN0aWNlcyBhbmQgZnV0dXJlIGNoYWxsZW5nZXMiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAwNy9zMTA3NTAtMDIwLTA0NDU3LXgiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vbGluay5zcHJpbmdlci5jb20vY29udGVudC9wZGYvMTAuMTAwNy9zMTA3NTAtMDIwLTA0NDU3LXgucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjAtMTEtMTYiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA3IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJIeWRyb2Jpb2xvZ2lhIiwiaXNzdWUiOjIxLCJ2b2x1bWUiOjg0NywiaW1wYWN0RmFjdG9yIjoiMy4wMCJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjQzNDMsImxhc3RQYWdlIjo0MzU3fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlNwcmluZ2VyIFNjaWVuY2UrQnVzaW5lc3MgTWVkaWEifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjk1LCJjaXRlZEJ5Q291bnQiOjU4fX0sInF1b3RlIjoiU2ltaWxhciBkZWdyYWRhdGlvbiBvZiBmcmVzaHdhdGVyIGVjb3N5c3RlbXMgaXMgb2JzZXJ2ZWQgd29ybGR3aWRlLCB3aXRoIHRoZSB0aW1lbGluZSBvZiBodW1hbiBwcmVzc3VyZSBkaWN0YXRlZCBieSB0cmVuZHMgaW4gdXJiYW5pemF0aW9uIGFuZCBpbnRlbnNpdmUgYWdyaWN1bHR1cmUgaW4gZGlmZmVyZW50IGdlb2dyYXBoaWMgcmVnaW9ucyAoZS5nLiwgVmVubmVtbyBldCBhbC4sIDIwMDkpLiBUaGUgcHJpbmNpcGFsIGRyaXZlciBvZiBlY29zeXN0ZW0gc3RyZXNzIGluIG1vc3QgYXF1YXRpYyBzeXN0ZW1zIGlzIGV1dHJvcGhpY2F0aW9uLCB0aGUgZXhjZXNzIGdyb3d0aCBvZiBhbGdhZSBkcml2ZW4gYnkgYW50aHJvcG9nZW5pYyBpbnB1dHMgb2YgdGhlIG1hY3JvbnV0cmllbnRzIG5pdHJvZ2VuIChOKSBhbmQgcGhvc3Bob3J1cyAoUCkgKFNtaXRoLCAyMDAzOyBCaXJrIGV0IGFsLiwgMjAyMCkuIE51dHJpZW50cyBhcmUgZW1pdHRlZCBmcm9tIGJvdGggcG9pbnQgYW5kIGRpZmZ1c2Ugc291cmNlcyBpbiB0aGUgaHVtYW4gZW52aXJvbm1lbnQgYW5kIGVudGVyIGFxdWF0aWMgc3lzdGVtcyB2aWEgc3VyZmFjZSBydW5vZmYsIGFzIHdlbGwgYXMgZ3JvdW5kLXdhdGVyIGFuZCBhdG1vc3BoZXJpYyBkZXBvc2l0aW9uIChQYWVybCwgMTk5NykuIEV1dHJvcGhpY2F0aW9uIG9mdGVuIGxlYWRzIHRvIHRoZSBkb21pbmFuY2Ugb2YgcHJpbWFyeSBwcm9kdWN0aW9uIGJ5IGN5YW5vYmFjdGVyaWEsIGluY2x1ZGluZyB0b3hpYyBnZW5lcmEgc3VjaCBhcyBEb2xpY2hvc3Blcm11bSwgQXBoYW5pem9tZW5vbiwgYW5kIE1pY3JvY3lzdGlzIChIdWlzbWFuIGV0IGFsLiwgMjAxOCksIGFzIHdlbGwgYXMgb3h5Z2VuIGRlcGxldGlvbiBhbmQgaGFiaXRhdCBkZWdyYWRhdGlvbiBpbiBsYWtlcyAoQXJlbmQgZXQgYWwuLCAyMDExKS4gQmVjYXVzZSBsYWtlcyBwcm92aWRlIGEgcmFuZ2Ugb2YgY3J1Y2lhbCBlY29zeXN0ZW0gc2VydmljZXMgKFdpbHNvbiAmIENhcnBlbnRlciwgMTk5OSksIHRoZXJlIGlzIGEgZ3Jvd2luZyBuZWVkIGZvciBtYW5hZ2VtZW50IGFwcHJvYWNoZXMgdGhhdCByZXN0b3JlIHdhdGVyIHF1YWxpdHkgaW4gZnJlc2h3YXRlciBzeXN0ZW1zLiBXaXRoaW4gdGhlIGZyYW1ld29yayBvZiBVbml0ZWQgTmF0aW9ucyBTdXN0YWluYWJsZSBEZXZlbG9wLW1lbnQgR29hbCA2IChDbGVhbiBXYXRlciBhbmQgU2FuaXRhdGlvbiksIHNldmVyYWwgZGlzdGluY3QgdGFyZ2V0cyByZWxhdGUgdG8gdGhlIGltcHJvdmVtZW50IG9mIHdhdGVyIHF1YWxpdHkgYW5kIHByb3RlY3Rpb24gb2YgYXF1YXRpYyBlY29zeXN0ZW1zIGJ5IDIwMzAgKFVuaXRlZCBOYXRpb25zLCAyMDE1KS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyOC4yOTg4NjQzNjQ2MjQwMjMsImxlZnQiOjguNTQ5MjA1NzgwMDI5Mjk3LCJ3aWR0aCI6MzkuMzcxNDk0MjkzMjEyODksImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MjkuOTkxMTM0NjQzNTU0Njg4LCJsZWZ0Ijo4LjU0OTIwNTc4MDAyOTI5Nywid2lkdGgiOjM5LjM2NTExOTkzNDA4MjAzLCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMxLjY4MzQwNjgyOTgzMzk4NCwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNjk2NjMyMzg1MjUzOSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozMy4zNjc5NzMzMjc2MzY3MiwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNzU2NzkwMTYxMTMyOCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNS4wNjAyNDkzMjg2MTMyOCwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNzA5NjQwNTAyOTI5NywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNi43NTI1MTc3MDAxOTUzMSwibGVmdCI6OC41NDkyMDg2NDEwNTIyNDYsIndpZHRoIjozOS4zNzY5NjA3NTQzOTQ1MywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC40MzcwODgwMTI2OTUzMSwibGVmdCI6OC41NDkyMDg2NDEwNTIyNDYsIndpZHRoIjozOS4zNjY1Njk1MTkwNDI5NywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0MC4xMjkzNTYzODQyNzczNDQsImxlZnQiOjguNTQ5MjA4NjQxMDUyMjQ2LCJ3aWR0aCI6MzkuMzY0Mzc5ODgyODEyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0MS44MjE2MjQ3NTU4NTkzNzUsImxlZnQiOjguNTQ5MjA4NjQxMDUyMjQ2LCJ3aWR0aCI6MzkuMzY1NjczMDY1MTg1NTUsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDMuNTA2MTk1MDY4MzU5Mzc1LCJsZWZ0Ijo4LjU0OTM4OTgzOTE3MjM2Mywid2lkdGgiOjM5LjM3MDM5MTg0NTcwMzEyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0NS4xOTg0NjM0Mzk5NDE0MDYsImxlZnQiOjguNTQ5NTcyOTQ0NjQxMTEzLCJ3aWR0aCI6MzkuMzY1ODQ0NzI2NTYyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ni44OTA3MzU2MjYyMjA3LCJsZWZ0Ijo4LjU0OTU3Mjk0NDY0MTExMywid2lkdGgiOjM5LjM3Mjk1NTMyMjI2NTYyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0OC41NzUzMDIxMjQwMjM0NCwibGVmdCI6OC41NDk1NzI5NDQ2NDExMTMsIndpZHRoIjozOS4zNzI0MjEyNjQ2NDg0NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1MC4yNjc1NzgxMjUsImxlZnQiOjguNTQ5NTc2NzU5MzM4Mzc5LCJ3aWR0aCI6MzkuMzcxMTI0MjY3NTc4MTI1LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjUxLjk1OTg0MjY4MTg4NDc2NiwibGVmdCI6OC41NDk1NzY3NTkzMzgzNzksIndpZHRoIjozOS4zNjc4MzU5OTg1MzUxNTYsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTMuNTc0MTE5NTY3ODcxMDk0LCJsZWZ0Ijo4LjU0OTU3Njc1OTMzODM3OSwid2lkdGgiOjM5LjM3MDc3NzEzMDEyNjk1LCJoZWlnaHQiOjEuMzM2ODk1NTg1MDYwMTE5Nn19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjU1LjI2NjM5MTc1NDE1MDM5LCJsZWZ0Ijo4LjU0OTc2MDgxODQ4MTQ0NSwid2lkdGgiOjM5LjM2NjU4MDk2MzEzNDc2NiwiaGVpZ2h0IjoxLjMzNjg5NTU4NTA2MDExOTZ9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1Ny4wMjg5NTM1NTIyNDYwOTQsImxlZnQiOjguNTQ5OTQ0ODc3NjI0NTEyLCJ3aWR0aCI6MzkuMzYyMDI2MjE0NTk5NjEsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTguNzEzNTIzODY0NzQ2MDk0LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjkuMjk5OTgyMDcwOTIyODUyLCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjYwLjQwNTc5MjIzNjMyODEyNSwibGVmdCI6MTAuNjIyNDU1NTk2OTIzODI4LCJ3aWR0aCI6MzcuMjg3ODY4NDk5NzU1ODYsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NjIuMDk4MDYwNjA3OTEwMTU2LCJsZWZ0Ijo4LjU0OTk0MzkyMzk1MDE5NSwid2lkdGgiOjM5LjM3MzMyNTM0NzkwMDM5LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjYzLjc4MjYzMDkyMDQxMDE1NiwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzMxMjY5ODM2NDI1OCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2NS40NzQ5MDY5MjEzODY3MiwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNjUxMTk5MzQwODIwMywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2Ny4xNjcxNzUyOTI5Njg3NSwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNjk0ODAxMzMwNTY2NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2OC43ODE0NDgzNjQyNTc4MSwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzczMzg0MDk0MjM4MywiaGVpZ2h0IjoxLjMzNjg5MTUzMTk0NDI3NX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjcwLjU0NDAxMzk3NzA1MDc4LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjM5LjM2OTMwODQ3MTY3OTY5LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjcyLjIzNjI4OTk3ODAyNzM0LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjM5LjM3MjAzMjE2NTUyNzM0NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo3My45MjA4NDUwMzE3MzgyOCwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzEzMDczNzMwNDY4NzUsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNDM0NCJ9fV0%3D. Eroded topsoil further amplifies turbidity, smothering benthic habitats and resuspending legacy sediments, perpetuating internal nutrient recycling (Haxthausen, 2021)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkthcmwgUmFzbXVzIEF1Z3VzdCIsImZhbWlseSI6IkhheHRoYXVzZW4ifX1dLCJpZHMiOnsiaXNzbiI6W10sImlzYm4iOltdLCJtYWciOiIzMTc3MTEzMjA3Iiwib3BlbmFsZXgiOiJXMzE3NzExMzIwNyJ9LCJ0aXRsZSI6IlNlZGltZW50IG1pY3JvYmlhbCBmdWVsIGNlbGxzIGZvciBsYWtlIHJlc3RvcmF0aW9uIiwiYWJzdHJhY3QiOiJFdXRyb3BoaWNhdGlvbiBvZiBsYWtlcyBpcyB1YmlxdWl0b3VzLCBhcyAzMC00MCAlIG9mIHRoZSB3b3JsZOKAmXMgbGFrZXMgYXJlIGV1dHJvcGhpYywgYW5kIGNvbmRpdGlvbnMgY29udGludW91c2x5IGRldGVyaW9yYXRpbmcuIEV1dHJvcGhpY2F0aW9uIGlzIGEgY29uZGl0aW9uIG9mIG92ZXJmZXJ0aWxpemF0aW9uLCB3aGVyZSBhbGwgcmVxdWlyZW1lbnRzIGZvciByYXBpZCBncm93dGggb2YgYWxnYWUgYXJlIG1ldCwgcmVzdWx0aW5nIGluIGFsZ2FsIGJsb29tcy4gVGhpcyBjYXVzZXMgYSBzZWxmLXN1c3RhaW5lZCwgdmljaW91cyBjeWNsZSB3aGVyZSBwb29yIGVjb2xvZ2ljYWwgY29uZGl0aW9ucyBhcmUgbWFpbnRhaW5lZCB1bnRpbCBudXRyaWVudHMgYXJlIGxvc3QgZnJvbSB0aGUgbGFrZSBvdmVyIG1hbnkgeWVhcnMuIExha2UgcmVzdG9yYXRpb24gc2Vla3MgdG8gYnJlYWsgdGhpcyB2aWNpb3VzIGN5Y2xlLiBJbmFkZXF1YXRlIGF2YWlsYWJpbGl0eSBvZiBhbnkgc2luZ2xlIG51dHJpZW50IGluIHdpbGwgbGltaXQgYWxnYWwgZ3Jvd3RoLCB0eXBpY2FsbHkgcGhvc3Bob3J1cyBpcyB0YXJnZXRlZC4gVGhlIHNjb3BlIG9mIHRoaXMgUGhEIHRoZXNpcyB3YXMgdG8gZGV2ZWxvcCwgdGVzdCBhbmQgb3B0aW1pemUgc2VkaW1lbnQgbWljcm9iaWFsIGZ1ZWwgY2VsbHMgKFNNRkMpIGFzIGEgbm92ZWwgbWV0aG9kIGZvciBsYWtlIHJlc3RvcmF0aW9uLiBUaGUgdGhlc2lzIHNwYW5zIHRoZSB0d28gbWFpbiB0b3BpY3Mgb2YgU01GQ3MgYW5kIGxha2UgcmVzdG9yYXRpb24gdGVjaG5pcXVlcy4gTGFrZSByZXN0b3JhdGlvbiB0ZWNobmlxdWVzIGFyZSByZXZpZXdlZCwgcHJvdmlkaW5nIHVuZGVyc3RhbmRpbmcgb2YgdGhlIHVuZGVybHlpbmcgbWVjaGFuaXNtcy4gU01GQ3MgYXJlIGFwcHJvYWNoZWQgZnJvbSBhbiBlbmdpbmVlcmluZyBwZXJzcGVjdGl2ZSBhbmQgYSBmb2N1cyBvbiBsYWtlIHJlbWVkaWF0aW9uLiBUaGUgc3BlY2lmaWMgb2JqZWN0aXZlcyB3ZXJlIHRvIHRlc3QgYSBsb3ctY29zdCBlbGVjdHJvZGUgbWF0ZXJpYWwgZm9yIGl0cyBkdXJhYmlsaXR5LCBwcmFjdGljYWxpdHksIGFuZCBwZXJmb3JtYW5jZS4gUGVyZm9ybWFuY2Ugd2FzIGFzc2Vzc2VkIGFzIHRoZSBhYmlsaXR5IHRvIGxpbWl0IHBob3NwaG9ydXMgcmVsZWFzZSBmcm9tIHNlZGltZW50cywgaW5oaWJpdCBtZXRoYW5lIHJlbGVhc2UsIGFuZCB0aGUgYWRkZWQgYmVuZWZpdHMgb2YgZGVncmFkaW5nIHBvbGx1dGFudHMsIGFuZCBnZW5lcmF0aW5nIGVsZWN0cmljaXR5LiBQcm9vZiBvZiBjb25jZXB0IHdhcyBzaG93biBpbiBsYWJvcmF0b3J5IHJlYWN0b3JzICgxIEwpLCB0aGUgZWZmZWN0IG9mIHVwc2NhbGluZyB0byAyMDAgTCBpbiBuYXR1cmFsIGVudmlyb25tZW50IHdhcyBzdHVkaWVkIGluIG91dGRvb3IgbWVzb2Nvc21zLCBhbmQgdGhlbiB0ZXN0ZWQgYXQgcGlsb3Qgc2NhbGUgaW4gYSBldXRyb3BoaWMgZnJlc2h3YXRlciBsYWtlLiBQaWxvdCBpbnN0YWxsYXRpb25zIHdlcmUgdGhlIGxhcmdlc3QgaW4gdGhlIHdvcmxkIG9mIHRoZWlyIGtpbmQgYW5kIHdlcmUgb3BlcmF0ZWQgZm9yIDIgeWVhcnMuIFN0YWlubGVzcyBzdGVlbCB3YXMgc2VsZWN0ZWQgZm9yIHRlc3RpbmcgYXMgZWxlY3Ryb2RlIG1hdGVyaWFsIGR1ZSB0byBpdHMgbG93IGNvc3QsIGR1cmFiaWxpdHksIGFuZCBhdmFpbGFiaWxpdHkuIFBhaXJzIG9mIGVsZWN0cm9kZXMgd2VyZSBlbWJlZGRlZCBpbiB0aGUgc2VkaW1lbnQgKHRoaXMgaXMgY2FsbGVkIHRoZSBhbm9kZSkgYW5kIHN1c3BlbmRlZCBpbiB0aGUgb3Zlcmx5aW5nIHdhdGVyICh0aGlzIGlzIGNhbGxlZCB0aGUgY2F0aG9kZSkuIFRoZSB0d28gZWxlY3Ryb2RlcyB3ZXJlIGNvbm5lY3RlZCBhY3Jvc3MgYW4gZXh0ZXJuYWwgcmVzaXN0b3Igd2l0aCBlbGVjdHJpY2FsIHdpcmVzLiBNaWNyb2JlcyBpbiB0aGUgc2VkaW1lbnQgdXNlIHRoZSBhbm9kZSBwYWlyIGFzIGVsZWN0cm9uIGFjY2VwdG9yLCBlbmFibGluZyByZXNwaXJhdGlvbiBpbiB0aGUgYW5veGljIHNlZGltZW50LiBFbGVjdHJvbnMgdHJhdmVsIGZyb20gdGhlIHNlZGltZW50IHZpYSB0aGUgZWxlY3RyaWNhbCBjb25uZWN0b3IgdG8gdGhlIG92ZXJseWluZyBjYXRob2RlIHdoZXJlIG94eWdlbiBpbiB0aGUgd2F0ZXIgaXMgdGhlIHRlcm1pbmFsIGVsZWN0cm9uIGFjY2VwdG9yIGNvbXBsZXRpbmcgdGhlIGNpcmN1aXQuIEFzIGFuIGFkZGVkIGJlbmVmaXQsIGRlZ3JhZGF0aW9uIG9mIHBvbGx1dGFudHMgdXNpbmcgU01GQyB3YXMgdGVzdGVkIGluIG1hcmluZSBoYXJib3VyIHNlZGltZW50cywgYnV0IG5vIGNvbmNsdXNpdmUgcmVzdWx0cyBvYnRhaW5lZC4gTGFib3JhdG9yeSByZWFjdG9ycyB3ZXJlIG9wZXJhdGVkIGZvciBtb3JlIHRoYW4gMjAwIGRheXMgdXNpbmcgcGV0cm9sZXVtIGFzIHNvbGUgY2FyYm9uIHNvdXJjZSwgY29uZmlybWluZyBvaWwgZGVncmFkYXRpb24gYnkgU01GQ3MuIEEgcmVtb3ZhbCByYXRlIG9mIDAuOSBnIHBldHJvbGV1bSBtLTMgZC0xIHJlYWN0b3Igdm9sdW1lIHdhcyBlc3RpbWF0ZWQgZnJvbSBtZWFzdXJlZCBlbGVjdHJpY2FsIG91dHB1dC4gVGhpcyB3YXMgZG9uZSBieSBhc3N1bXB0aW9uIG9mIHRoZSBjaGVtaWNhbCByZWFjdGlvbiBvY2N1cnJpbmcsIGZyb20gdGhpcyB0aGUgbnVtYmVyIG9mIGVsZWN0cm9ucyB0cmFuc2ZlcnJlZCBmcm9tIG9pbCB0byB3YXRlciB3YXMga25vd24gYW5kIGNvdWxkIGJlIGNvcnJlbGF0ZWQgdG8gdGhlIG1lYXN1cmVkIGZsb3cgb2YgZWxlY3Ryb25zIChjdXJyZW50KSB0aHJvdWdoIHRoZSBleHRlcm5hbCBjaXJjdWl0LiBTdGFpbmxlc3Mgc3RlZWwgZWxlY3Ryb2RlcyB3ZXJlIG9wZXJhdGVkIGluIGZyZXNod2F0ZXIgbGFrZSBlbnZpcm9ubWVudHMgZm9yIG1vcmUgdGhhbiA0IHllYXJzIHdpdGggbWluaW1hbCBkYW1hZ2UgdG8gdGhlIG1hdGVyaWFsLiBUaGUgZWxlY3Ryb2RlIGlzIHRoZSBtYWluIG1hdGVyaWFsIGNvc3QgaW4gc3VjaCBzeXN0ZW1zLiBIb3dldmVyLCBBSVNJMzE2IHN0YWlubGVzcyBzdGVlbCBzaW1pbGFyIHRvIHdoYXQgd2FzIHVzZWQgaW4gdGhpcyB0aGVzaXMgd2FzIGF0IHRoZSB0aW1lIG9mIHdyaXRpbmcgYXZhaWxhYmxlIGF0IGJ1bGsgY29zdCBvZiAwLjA5MSDigqwgbS0yLCBjb3JyZXNwb25kaW5nIHRvIDkxMCDigqwgYWNyZS0xLCBzaWduaWZpY2FudGx5IGxvd2VyIHRoYW4gb3RoZXIgY29tbW9uIG1hdGVyaWFscy4gQ2F0aG9kZXMgYXQgbGFyZ2Ugc2NhbGUgY291bGQgYmUgbW92ZWQgaG9yaXpvbnRhbGx5IGFuZCB2ZXJ0aWNhbGx5IHdpdGggbWlub3IgaW1wYWN0IHRvIHBlcmZvcm1hbmNlLiBQaWxvdCBzY2FsZSBTTUZDcyBnZW5lcmF0ZWQgMC4zODMgbVcgbS0yIHN1c3RhaW5lZCBhbmQgMjcuNCBtVyBtLTIgcGVhayBvdXRwdXQuIFRoZSBzeXN0ZW0gd2FzIGxpbWl0ZWQgYnkgdGhlIGNhdGhvZGUgc3VyZmFjZSBhcmVhLCBtZWFuaW5nIG11bHRpLWxheWVyIGNhdGhvZGVzIHNob3VsZCBiZSB0ZXN0ZWQgaW4gdGhlIGZ1dHVyZSBpZiBlbGVjdHJpY2FsIHBvd2VyIHByb2R1Y3Rpb24gaXMgdGhlIGdvYWwuIFRoZSBjYXRob2RlIHNob3VsZCBiZSBwbGFjZWQgYXQgYSBkZXB0aCB0aGF0IGVuc3VyZXMgaGlnaCBkaXNzb2x2ZWQgb3h5Z2VuIGNvbmNlbnRyYXRpb25zIHRvIGF2b2lkIHRoZSBzeXN0ZW0gdG8gYmUgbGltaXRlZC4gSG93ZXZlciwgdG9vIGhpZ2ggZWxlY3RyaWNpdHkgZ2VuZXJhdGlvbiB3YXMgZm91bmQgY291bnRlcnByb2R1Y3RpdmUgZm9yIHJlbWVkaWF0aW9uIHB1cnBvc2VzIGJlY2F1c2UgdGhpcyB3b3VsZCBpbmNyZWFzZSBzZWRpbWVudCB0dXJub3ZlciByYXRlLCByZWxlYXNpbmcgcG9sbHV0YW50cyBzdG9yZWQgaW4gdGhlIHNlZGltZW50cy4gUCBiaW5kaW5nIHdhcyBub3QgZm91bmQgdG8gcmVxdWlyZSBoaWdoIHR1cm5vdmVyIG9mIHNlZGltZW50IG9yZ2FuaWMgbWF0dGVyLiBUaGUgdHVybm92ZXIgY2FuIGJlIGNvbnRyb2xsZWQgYnkgY2hhbmdpbmcgdGhlIGV4dGVybmFsIHJlc2lzdGFuY2UuIEhpZ2ggcmVzaXN0YW5jZSBsaW1pdHMgdGhlIGVsZWN0cmljYWwgcGVyZm9ybWFuY2UsIGxvdyByZXNpc3RhbmNlIGluY3JlYXNlcyBzZWRpbWVudCB0dXJub3ZlciBhbmQgcmlza3MgZGVzdGFiaWxpemluZyB0aGUgc3lzdGVtLiBUbyBhY2hpZXZlIHJlZHVjZWQgY29uY2VudHJhdGlvbnMgb2YgcGhvc3BoYXRlIGluIHRoZSB3YXRlciBjb2x1bW4sIGFub2RlcyBtdXN0IGJlIHBsYWNlZCBpbiB0aGUgc2VkaW1lbnQgc3VyZmFjZSBzaW5jZSBhIGZldyBtaWxsaW1ldHJlcyBvZiBzZWRpbWVudCBtYXkgY29udGFpbiBhbmQgcmVsZWFzZSBlbm91Z2ggUCB0byBzdXN0YWluIGV1dHJvcGhpY2F0aW9uLiBQaG9zcGhhdGUgd2FzIG1lYXN1cmVkIGluIHRoZSB3YXRlciBjb2x1bW4gbmVhciB0aGUgc2VkaW1lbnQsIGluIHN5c3RlbXMgd2l0aCBhbmQgd2l0aG91dCBlbGVjdHJvZGVzLiBQaG9zcGhhdGUgY29uY2VudHJhdGlvbnMgd2VyZSBzdWNjZXNzZnVsbHkgcmVkdWNlZCBieSB1cCB0byA5NSAlIGluIDItY2hhbWJlciBsYWJvcmF0b3J5IHJlYWN0b3JzLCBhbmQgYnkgdXAgdG8gOTggJSBpbiB0aGUgcGlsb3QgaW5zdGFsbGF0aW9uLiBJbiB0aGUgcGlsb3QgZXhwZXJpbWVudCwgcGhvc3BoYXRlIGNvbmNlbnRyYXRpb25zIGFib3ZlIGVsZWN0cm9kZXMgd2VyZSBsb3dlciBpbiBhbGwgbWVhc3VyZW1lbnRzLCBjb21wYXJlZCB0byBjb250cm9scy4gU2VkaW1lbnQgcmVkb3ggcG90ZW50aWFscyBpbmNyZWFzZWQgZnJvbSAtNDAwIG1WIHZzIHN0YW5kYXJkIGh5ZHJvZ2VuIGVsZWN0cm9kZSAoU0hFKSB0byAtMTc1IG1WIHZzIFNIRSwgY29ycmVzcG9uZGluZyB0byBhIG1vdmUgZnJvbSBhIG1ldGhhbm9nZW5ldGljIGRvbWFpbiB0byBzdWxwaHVyIHJlZHVjaW5nLiBBbmFseXNpcyBvZiB0aGUgbWljcm9iaWFsIGNvbW11bml0eSBpbmRpY2F0ZWQgZW5oYW5jZWQgc3VscGh1ciBjeWNsaW5nIHdoZXJlIGVsZWN0cm9kZXMgb3hpZGl6ZSBzdWxwaGlkZSB3aGljaCB0aGUgYmFjdGVyaWEgYWdhaW4gcmVkdWNlIHRvIG9idGFpbiBlbmVyZ3kuIEl0IHdhcyBoeXBvdGhlc2lzZWQgdGhhdCBTTUZDIHByb21vdGUgaGlnaGVyIHJlZG94IHBvdGVudGlhbHMgYnkgZmFjaWxpdGF0aW5nIG94aWRhdGlvbiBvZiBzdWxwaHVyIGFuZCBpcm9uIHNwZWNpZXMsIGluaGliaXRpbmcgSDJTIGZvcm1hdGlvbiBhbmQgRmUgc2NhdmVuZ2luZyBieSBGZVMgZm9ybWF0aW9uLCB0aHVzIGFsbG93aW5nIGZvciBQIGJpbmRpbmcuIEEgbG93ZXIgcmVsYXRpdmUgYWJ1bmRhbmNlIG9mIG1ldGhhbm9nZW5zIG9uIGVsZWN0cm9kZXMgY29tcGFyZWQgdG8gY29udHJvbHMgc3VnZ2VzdHMgc3VwcHJlc3Npb24gb3Igb3V0Y29tcGV0aW5nIG9mIG1ldGhhbm9nZW5zLlF1YW50aWZpY2F0aW9uIG9mIG1ldGhhbmUgcHJvZHVjdGlvbiB3YXMgZG9uZSBvbiB0aGUgYmFzaXMgb2YgYSBDSDQ6Q08yIHJhdGlvIHRvIGFsbG93IGNvbXBhcmFiaWxpdHkgYmV0d2VlbiBzeXN0ZW1zLiBNZXRoYW5lIGVtaXNzaW9uIHdhcyByZWR1Y2VkIGJ5IHVwIHRvIDk5ICUgYW5kIG1haW50YWluZWQgaW5oaWJpdGlvbiBmb3Igc2V2ZXJhbCBtb250aHMgaW4gbGFib3JhdG9yeSBzeXN0ZW1zIHdpdGggZWxlY3Ryb2Rlcy4gSW4gdGhlIHBpbG90IGluc3RhbGxhdGlvbnMgbm8gZ2FzIHNhbXBsZXMgY291bGQgYmUgY29sbGVjdGVkLCBhbmQgbm8gQ08yOkNINCByYXRpb3Mgd2VyZSBvYnRhaW5lZC4gQWVyYXRpb24gaXMgdGhlIGN1cnJlbnQgcmVzdG9yYXRpb24gdGVjaG5pcXVlIHRoYXQgbW9zdCBjbG9zZWx5IHJlc2VtYmxlcyB0aGUgcHJvcG9zZWQgU01GQyB0ZWNobmlxdWUuIEFlcmF0aW9uIGhhcyBwcmV2aW91c2x5IGZhaWxlZCB0byByZWR1Y2UgUCByZWxlYXNlIGluIHNldmVyYWwgc2VwYXJhdGUgaW52ZXN0aWdhdGlvbnMuIFNNRkNzIGFzIGEgcmVtZWRpYXRpb24gdG9vbCB3YXMgZm91bmQgdG8gb2ZmZXIgc2V2ZXJhbCBiZW5lZml0cyBvdmVyIGNvbnZlbnRpb25hbCBtZXRob2RzOiBTTUZDcyBvZmZlcnMgY29udGludW91cyBlZmZlY3QsIHdoaWxlIG90aGVyIHRlY2huaXF1ZXMgcmVxdWlyZSBjb250aW51b3VzIG9yIHJlcGVhdGVkIHRyZWF0bWVudC4gU01GQ3MgY2FuIGJlIGNvbnRyb2xsZWQgaW4gcmVhbC10aW1lIGJ5IGFsdGVyaW5nIHRoZSBleHRlcm5hbCByZXNpc3RvciBhbmQgY2FuIGJlIGVhc2lseSBtb25pdG9yZWQgYnkgbWVhc3VyaW5nIHRoZSBwcm9kdWNlZCBlbGVjdHJpY2FsIG91dHB1dC4gSWYgaW1wbGVtZW50ZWQgYXQgbGFyZ2Ugc2NhbGUgaXQgaXMgZXhwZWN0ZWQgdGhhdCB0aGUgZWxlY3Ryb2RlIGZpZWxkcyB3aWxsIGhlbHAgbWFpbnRhaW4gbG93IHBob3NwaGF0ZSBjb25jZW50cmF0aW9ucyBpbiB0aGUgd2F0ZXIgZm9yIGRlY2FkZXMgd2l0aCBtaW5pbWFsIGludGVydmVudGlvbiwgYW5kIGFmdGVyIHNvbWUgdGltZSwgYmVudGhpYyBwbGFudHMgYXJlIGV4cGVjdGVkIHRvIHJldHVybiwgdGh1cyBtYWludGFpbmluZyB0aGUgYmFsYW5jZSBoZW5jZSBmb3J0aC4iLCJ1cmwiOiJodHRwczovL2xvY2FsLmZvcnNrbmluZ3Nwb3J0YWwuZGsvbG9jYWwvZGtpLWNnaS93cy9jcmlzLWxpbms%2Fc3JjPWR0dSZpZD1kdHUtZDY2ZDMzNzAtMjU2Zi00ODg1LWEwOWUtMzM2YTkyNGY0YmVkJnRpPVNlZGltZW50JTIwbWljcm9iaWFsJTIwZnVlbCUyMGNlbGxzJTIwZm9yJTIwbGFrZSUyMHJlc3RvcmF0aW9uIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2JhY2tlbmQub3JiaXQuZHR1LmRrL3dzL2ZpbGVzLzI0ODA2OTU4OS9UaGVzaXNfb25saW5lX3ZlcnNpb25fS2FybF9IYXh0aGF1c2VuLnBkZiIsImlzc3VlZERhdGUiOiIyMDIxLTAxLTAxIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0wOCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiUmVzZWFyY2ggUG9ydGFsIERlbm1hcmsiLCJpbXBhY3RGYWN0b3IiOiIwLjA0In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6NzZ9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiVGVjaG5pY2FsIFVuaXZlcnNpdHkgb2YgRGVubWFyayJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MCwiY2l0ZWRCeUNvdW50IjoxfX0sInF1b3RlIjoiRXh0ZXJuYWwgaW5wdXRzIGluY3JlYXNlIFAgY29uY2VudHJhdGlvbiBpbiAgdGhlIHdhdGVyLCBjYXVzaW5nIGFsZ2FsIGJsb29tcyB0aGF0IHNoYWRvdyB0aGUgbGFrZSBib3R0b20sIGFuZCBiZW50aGljIG1hYy1yb3BoeXRlcyByZWNlZGUuIFByZWRhdG9yeSBmaXNoIGFyZSB1bmFibGUgdG8gaHVudCwgYW5kIHRoZWlyIGZvb2Qgc291cmNlIGlzICBkaXNydXB0ZWQgc3VjaCBhcyBmaXNoIGZlZWRpbmcgb24gYmVudGhpYyBpbnZlcnRlYnJhdGVzIGFuZCBib3R0b20tZHdlbGxpbmcgYW5pbWFscyBhcmUgc3VmZm9jYXRlZCAoVmFuZGVyIFphbmRlbiBhbmQgVmFkZWJvbmNvZXVyIDIwMDIpLCBzbyB0aGF0IHBsYW5rdGl2b3JvdXMgZmlzaCBzcGVjaWVzIGRvbWluYXRlLiBUaGlzIGluY3JlYXNlcyBwcmVkYXRpb24gb24gem9vIHBsYW5rdG9uICBhbmQgcmVkdWNlcyBncmF6aW5nIG9uIGFsZ2FlLiBBbGdhZSBwcm9saWZlcmF0ZSwgZGllLCBhbmQgc2luayB0byB0aGUgYm90dG9tLCAgY29uc3VtaW5nIG94eWdlbiB3aGVuIGRlZ3JhZGVkLCB3aGljaCBsZWFkcyB0byBveHlnZW4gZGVmaWNpdCBpbiB0aGUgYm90LXRvbSwga2lsbGluZyBmaXNoIChXdXJ0c2JhdWdoIGV0IGFsLiAyMDE5KS4gRmlndXJlIDI6IFZpY2lvdXMgY3ljbGVzIG9mIGV1dHJvcGhpY2F0aW9uLiBMZWZ0OiBJbnRlcmxpbmtlZCBwcm9jZXNzZXMgc3VzdGFpbmluZyBldXRyb3BoaS1jYXRpb24gaW4gYSBsYWtlLiBCbHVlOiBwcm9jZXNzZXMgaW4gd2F0ZXIsIGdyZWVuOiBwcm9jZXNzZXMgaW52b2x2aW5nIG1pY3JvYWxnYWUsIGdyZXk6IHNlZC1pbWVudCByZWxhdGVkIHByb2Nlc3NlcywgeWVsbG93OiBwcm9jZXNzZXMgaW52b2x2aW5nIGZhdW5hLiBSaWdodCwgZGFzaGVkIGxpbmU6IFRoZSBvcmRlciBpbiB3aGljaCBlbGVjdHJvbiBhY2NlcHRvcnMgYXJlIGNvbnN1bWVkIGluIHRoZSBhYnNlbmNlIG9mIE8yIHNob3duIGFzIGNvbmNlbnRyYXRpb24gdmVyc3VzICBkZXB0aCBpbiBzZWRpbWVudCwgYW5kIGF0IHdoaWNoIHBvdGVudGlhbCB0aGlzIG9jY3VycyAoQmVybmVyIDE5ODEsIGJ5IGNvdXJ0ZXN5IG9mIHRoZSBzb2NpZXR5ICBmb3Igc2VkaW1lbnRhcnkgZ2VvbG9neSkuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxNywiYmJveCI6eyJ0b3AiOjc5LjkzMzE1MTI0NTExNzE5LCJsZWZ0IjoxNC4yOTU1MDM2MTYzMzMwMDgsIndpZHRoIjo3MS45Mzg5MzQzMjYxNzE4OCwiaGVpZ2h0IjoyLjA1MzQ3NzI4NzI5MjQ4MDV9fSx7InBhZ2VOdW1iZXIiOjE3LCJiYm94Ijp7InRvcCI6ODEuOTcxMzQzOTk0MTQwNjIsImxlZnQiOjE0LjI5NTUwMzYxNjMzMzAwOCwid2lkdGgiOjcxLjMzMzAwMDE4MzEwNTQ3LCJoZWlnaHQiOjIuMDUzNDc3Mjg3MjkyNDgwNX19LHsicGFnZU51bWJlciI6MTcsImJib3giOnsidG9wIjo4NC4wMjM3OTYwODE1NDI5NywibGVmdCI6MTQuMjk1NTAzNjE2MzMzMDA4LCJ3aWR0aCI6NzEuODEyMDE5MzQ4MTQ0NTMsImhlaWdodCI6Mi4wNTM0NzcyODcyOTI0ODA1fX0seyJwYWdlTnVtYmVyIjoxNywiYmJveCI6eyJ0b3AiOjg2LjA2MTk5NjQ1OTk2MDk0LCJsZWZ0IjoxNC4yOTU1MDM2MTYzMzMwMDgsIndpZHRoIjo3MS45NTczNTkzMTM5NjQ4NCwiaGVpZ2h0IjoyLjA1MzQ3NzI4NzI5MjQ4MDV9fSx7InBhZ2VOdW1iZXIiOjE4LCJiYm94Ijp7InRvcCI6OS45MjY0ODc5MjI2Njg0NTcsImxlZnQiOjE0LjI5NTUwMzYxNjMzMzAwOCwid2lkdGgiOjcxLjk3NDA2MDA1ODU5Mzc1LCJoZWlnaHQiOjEwLjIzNDc3NzQ1MDU2MTUyM319LHsicGFnZU51bWJlciI6MTgsImJib3giOnsidG9wIjo0NC43MzQwNDMxMjEzMzc4OSwibGVmdCI6MTQuMjk1NTAzNjE2MzMzMDA4LCJ3aWR0aCI6NzEuODkzNDMyNjE3MTg3NSwiaGVpZ2h0IjoxMC40MDMyODk3OTQ5MjE4NzV9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI5MyJ9fV0%3D.

Figure 3: Agricultural Runoff

Fertilizer overuse disrupts nutrient stoichiometry, with phosphorus often exceeding nitrogen due to subsidized phosphatic inputs, shifting lakes from N-limitation to P-excess conditions that paradoxically favor nitrogen-fixing cyanobacteria like Dolichospermum and Aphanizomenon (Haq et al., 2024; Nõges et al., 2020; Swann et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRpaW5hIiwiZmFtaWx5IjoiTsO1Z2VzIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hc2ltZSIsImZhbWlseSI6IkphbmF0aWFuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlJlZXQiLCJmYW1pbHkiOiJMYXVnYXN0ZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQZWV0ZXIiLCJmYW1pbHkiOiJOw7VnZXMifX1dLCJpZHMiOnsiaXNzbiI6WyIyMzUxLTk4OTQiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMTYvai5nZWNjby4yMDIwLmUwMTM2OSIsIm1hZyI6IjMxMDU0NDYwOTAiLCJvcGVuYWxleCI6IlczMTA1NDQ2MDkwIiwiZXhhIjoiaHR0cHM6Ly93d3cuc2NpZW5jZWRpcmVjdC5jb20vc2NpZW5jZS9hcnRpY2xlL3BpaS9TMjM1MTk4OTQyMDMwOTEwMCJ9LCJ0aXRsZSI6IlBvc3Qtc292aWV0IGNoYW5nZXMgaW4gbml0cm9nZW4gYW5kIHBob3NwaG9ydXMgc3RvaWNoaW9tZXRyeSBpbiB0d28gbGFyZ2Ugbm9uLXN0cmF0aWZpZWQgbGFrZXMgYW5kIHRoZSBpbXBhY3Qgb24gcGh5dG9wbGFua3RvbiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2ouZ2VjY28uMjAyMC5lMDEzNjkiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2ouZ2VjY28uMjAyMC5lMDEzNjkiLCJpc3N1ZWREYXRlIjoiMjAyMC0xMS0yMCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTEiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6Ikdsb2JhbCBFY29sb2d5IGFuZCBDb25zZXJ2YXRpb24iLCJ2b2x1bWUiOjI0LCJpbXBhY3RGYWN0b3IiOiIzLjU5In0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkVsc2V2aWVyIEJWIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo3MSwiY2l0ZWRCeUNvdW50IjoxNH19LCJxdW90ZSI6IlJhbmRvbSBGb3Jlc3QgYW5hbHlzaXMgaW5kaWNhdGVkIGFuIGltcG9ydGFudCByb2xlIG9mIGxpZ2h0IGxpbWl0YXRpb24gaW4gYm90aCBsYWtlcy4gQ29uc3RhbnRseSBsb3dlciBsZXZlbHMgb2YgTiBpbiB0aGUgZGVlcGVyIExha2UgUGVpcHNpIGZhdm91cmVkIE4yLWZpeGluZyBzcGVjaWVzLCB3aGljaCwgYXMgYSBwYXJhZG94LCBiZWNhbWUgUC1saW1pdGVkLiBBcyBjbGltYXRlIHdhcm1pbmcgcmVpbmZvcmNlcyBldXRyb3BoaWNhdGlvbiBwaGVub21lbmEgaW4gbGFrZXMgYnkgaW5jcmVhc2luZyBpbnRlcm5hbCBudXRyaWVudCBsb2FkaW5nIGFuZCBmYXZvdXJpbmcgYmxvb20tZm9ybWluZyBjeWFub2JhY3RlcmlhLCBtb3JlIHN0cmluZ2VudCBtZWFzdXJlcyB3b3VsZCBiZSBuZWVkZWQgdG8gZnVydGhlciBsaW1pdCBudXRyaWVudCBsb2FkcyAoZXNwZWNpYWxseSB0aGF0IG9mIFApIHRvIGxha2VzIHRocm91Z2ggaW1wcm92ZWQgd2FzdGV3YXRlciB0cmVhdG1lbnQgYW5kIGluY3JlYXNlZCBlZmZpY2llbmN5IG9mIGZlcnRpbGlzZXIgYXBwbGljYXRpb24uIExvbmctdGVybSBjaGFuZ2VzIGluIG51dHJpZW50IGxvYWRpbmcgYW5kIHN0b2ljaGlvbWV0cnkgaW4gd2F0ZXIgYm9kaWVzIHJlZmxlY3QgY2hhbmdlcyBpbiBhZ3JpY3VsdHVyYWwgcHJhY3RpY2VzIGFuZCB3YXN0ZXdhdGVyIHRyZWF0bWVudC4ifSx7Im9yaWdpbiI6NCwid29yayI6eyJ0eXBlIjo0LCJzdWJ0eXBlIjp7fSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiWnVsZnFhcnVsIiwiZmFtaWx5IjoiSGFxIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNub2JlciIsImZhbWlseSI6Iklyc2hhZCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBem1hdCBBbGFtIiwiZmFtaWx5IjoiS2hhbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTaGVpa2ggRi4iLCJmYW1pbHkiOiJBaG1hZCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTaG93a2VlbiBNdXphbWlsIiwiZmFtaWx5IjoiQmFzaGlyIn19XSwiaWRzIjp7Imlzc24iOlsiMjYzMi0wNTE3Il0sImlzYm4iOltdLCJkb2kiOiIxMC41NzcyL2ludGVjaG9wZW4uMTAwNjU3OCIsIm9wZW5hbGV4IjoiVzQ0MDQ1MjkyMDMifSwidGl0bGUiOiJBZHZhbmNlcyBpbiBNYW5hZ2luZyBOaXRyb2dlbiBhbmQgUGhvc3Bob3J1cyBFbWlzc2lvbnMgaW4gUnVtaW5hbnRzOiBBIEhvbGlzdGljIEFwcHJvYWNoIiwiYWJzdHJhY3QiOiLigJxCYWxhbmNpbmcgTnV0cml0aW9uIGFuZCBFbnZpcm9ubWVudGFsIFN1c3RhaW5hYmlsaXR5OiBBZHZhbmNlcyBpbiBNYW5hZ2luZyBOaXRyb2dlbiBhbmQgUGhvc3Bob3J1cyBFbWlzc2lvbnMgaW4gUnVtaW5hbnRz4oCdIGV4cGxvcmVzIHRoZSBpbnRyaWNhdGUgcmVsYXRpb25zaGlwIGJldHdlZW4gcnVtaW5hbnQgbnV0cml0aW9uIGFuZCBlbnZpcm9ubWVudGFsIHN0ZXdhcmRzaGlwLiBJdCBkZWx2ZXMgaW50byB0aGUgZHluYW1pY3Mgb2Ygbml0cm9nZW4gYW5kIHBob3NwaG9ydXMgbWV0YWJvbGlzbSBpbiBydW1pbmFudHMsIGFkZHJlc3NpbmcgdGhlIGVudmlyb25tZW50YWwgcmVwZXJjdXNzaW9ucyBvZiB0aGVpciBlbWlzc2lvbnMuIFRoZXNlIGVtaXNzaW9ucywgcHJpbWFyaWx5IGluIHRoZSBmb3JtIG9mIGFtbW9uaWEgYW5kIHBob3NwaGF0ZXMsIHBvc2Ugc2lnbmlmaWNhbnQgZW52aXJvbm1lbnRhbCBjaGFsbGVuZ2VzLCBjb250cmlidXRpbmcgdG8gYWlyIGFuZCB3YXRlciBwb2xsdXRpb24sIHNvaWwgZGVncmFkYXRpb24sIGFuZCBjbGltYXRlIGNoYW5nZS4gVGhlIGNoYXB0ZXIgaGlnaGxpZ2h0cyBpbm5vdmF0aXZlIHN0cmF0ZWdpZXMgYW5kIHRlY2hub2xvZ2ljYWwgYnJlYWt0aHJvdWdocyBhaW1lZCBhdCBtaXRpZ2F0aW5nIG5pdHJvZ2VuIGFuZCBwaG9zcGhvcnVzIGV4Y3JldGlvbiwgY3J1Y2lhbCBmb3IgcmVkdWNpbmcgZW52aXJvbm1lbnRhbCBwb2xsdXRpb24uIEl0IG9mZmVycyBpbnNpZ2h0cyBpbnRvIGRpZXRhcnkgYWRqdXN0bWVudHMsIGZlZWQgYWRkaXRpdmVzLCBhbmQgYmVzdCBwcmFjdGljZXMgZm9yIHN1c3RhaW5hYmxlIHJ1bWluYW50IHByb2R1Y3Rpb24uIEtleSBhcmVhcyBvZiBmb2N1cyBpbmNsdWRlIGRpZXRhcnkgYWRqdXN0bWVudHMgdGFpbG9yZWQgdG8gb3B0aW1pemUgbnV0cmllbnQgdXRpbGl6YXRpb24gYW5kIHJlZHVjZSB3YXN0ZSwgdGhlIHV0aWxpemF0aW9uIG9mIGZlZWQgYWRkaXRpdmVzIHRvIGVuaGFuY2UgbnV0cmllbnQgYWJzb3JwdGlvbiBhbmQgbWluaW1pemUgZXhjcmV0aW9uLCBhbmQgdGhlIGltcGxlbWVudGF0aW9uIG9mIGJlc3QgbWFuYWdlbWVudCBwcmFjdGljZXMgdG8gcHJvbW90ZSBzdXN0YWluYWJsZSBydW1pbmFudCBwcm9kdWN0aW9uLiBCeSBpbnRlZ3JhdGluZyBzY2llbnRpZmljIGZpbmRpbmdzIHdpdGggcHJhY3RpY2FsIGFwcGxpY2F0aW9ucywgdGhpcyByZXZpZXcgdW5kZXJzY29yZXMgdGhlIGltcG9ydGFuY2Ugb2YgYWNoaWV2aW5nIGhhcm1vbnkgYmV0d2VlbiBudXRyaXRpb24gb3B0aW1pemF0aW9uIGFuZCBlbnZpcm9ubWVudGFsIHJlc3BvbnNpYmlsaXR5IGluIG1vZGVybiBhZ3JpY3VsdHVyZS4gVGhyb3VnaCB0aGlzIGNvbXByZWhlbnNpdmUgYW5hbHlzaXMsIHRoZSBjaGFwdGVyIGFpbXMgZm9yIGJhbGFuY2luZyB0aGUgZHVhbCBnb2FscyBvZiBvcHRpbWl6aW5nIHJ1bWluYW50IG51dHJpdGlvbiBhbmQgbWluaW1pemluZyBlbnZpcm9ubWVudGFsIGltcGFjdCwgdGhlcmVieSBjb250cmlidXRpbmcgdG8gbW9yZSBzdXN0YWluYWJsZSBhbmQgcmVzaWxpZW50IGFncmljdWx0dXJhbCBzeXN0ZW1zLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC41NzcyL2ludGVjaG9wZW4uMTAwNjU3OCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cuaW50ZWNob3Blbi5jb20vY2l0YXRpb24tcGRmLXVybC8xMTg3ODE0IiwiaXNzdWVkRGF0ZSI6IjIwMjQtMTEtMTUiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA5IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJWZXRlcmluYXJ5IG1lZGljaW5lIGFuZCBzY2llbmNlIiwiaW1wYWN0RmFjdG9yIjoiMC45OSJ9LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJJbnRlY2hPcGVuIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo1MiwiY2l0ZWRCeUNvdW50IjoxfX0sInF1b3RlIjoiV2F0ZXIgUXVhbGl0eSBDaGFuZ2VzIOKAoiBDbGFyaXR5IGFuZCBBbGdhbCBCbG9vbXM6IFdhdGVyIGNsYXJpdHkgbWF5IGRlY3JlYXNlIHNsaWdodGx5IGR1ZSB0byB0aGUgaW5pdGlhbCBhbGdhbCBibG9vbXMuIEhvd2V2ZXIsIHRoaXMgcGhhc2UgaXMgb2Z0ZW4gc3RpbGwgY2hhcmFjdGVyaXplZCBieSByZWxhdGl2ZWx5IGdvb2Qgd2F0ZXIgcXVhbGl0eS4g4oCiIEJpb2RpdmVyc2l0eTogVGhlIGVjb3N5c3RlbSBtaWdodCBzdGlsbCBzdXBwb3J0IGEgZGl2ZXJzZSByYW5nZSBvZiBhcXVhdGljIG9yZ2FuaXNtcywgaW5jbHVkaW5nIGZpc2ggYW5kIGludmVydGVicmF0ZXMuIDMuMS40LjIgUGhhc2UgMjogTWVzb3Ryb3BoaWMgdG8gZXV0cm9waGljIHRyYW5zaXRpb24gMS4gTnV0cmllbnQgSW1iYWxhbmNlIOKAoiBQaG9zcGhvcnVzIHZzLiBOaXRyb2dlbjogQXMgYWdyaWN1bHR1cmFsIHByYWN0aWNlcyBpbnRlbnNpZnkgYW5kIHVyYmFuaXphdGlvbiBpbmNyZWFzZXMsIHBob3NwaG9ydXMgaW5wdXRzIChlLmcuLCBmcm9tIGZlcnRpbGl6ZXJzIGFuZCBkZXRlcmdlbnRzKSBvZnRlbiBleGNlZWQgdGhvc2Ugb2Ygbml0cm9nZW4uIFRoaXMgaW1iYWxhbmNlIGNhbiBkcml2ZSBzaWduaWZpY2FudCBjaGFuZ2VzIGluIHdhdGVyIGNoZW1pc3RyeSBhbmQgZWNvc3lzdGVtIGR5bmFtaWNzLiAyLiBBbGdhbCBCbG9vbSBJbnRlbnNpZmljYXRpb24g4oCiIEN5YW5vYmFjdGVyaWEgRG9taW5hbmNlOiBFeGNlc3MgcGhvc3Bob3J1cyBjYW4gbGVhZCB0byB0aGUgZG9taW5hbmNlIG9mIGN5YW5vYmFjdGVyaWEgKGJsdWUtZ3JlZW4gYWxnYWUpLCB3aGljaCBjYW4gb3V0Y29tcGV0ZSBvdGhlciBhbGdhZSBhbmQgYXF1YXRpYyBwbGFudHMuIFRoZXNlIGJsb29tcyBjYW4gYmUgdG94aWMgYW5kIGhhcm1mdWwgdG8gYXF1YXRpYyBsaWZlLiDigKIgSHlwb3hpYTogVGhlIGRlY29tcG9zaXRpb24gb2YgbGFyZ2UgYWxnYWwgYmxvb21zIGNvbnN1bWVzIG94eWdlbiwgbGVhZGluZyB0byBoeXBveGljIChsb3cgb3h5Z2VuKSBvciBhbm94aWMgKG5vIG94eWdlbikgY29uZGl0aW9ucywgcGFydGljdWxhcmx5IGluIGRlZXBlciB3YXRlcnMuIDMuIFdhdGVyIFF1YWxpdHkgRGV0ZXJpb3JhdGlvbiDigKIgRGVjcmVhc2VkIENsYXJpdHk6IFdhdGVyIGNsYXJpdHkgc2lnbmlmaWNhbnRseSBkZWNyZWFzZXMgZHVlIHRvIGRlbnNlIGFsZ2FsIGdyb3d0aC4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NjcuMjYxMDAxNTg2OTE0MDYsImxlZnQiOjE1LjU4MzQ1ODkwMDQ1MTY2LCJ3aWR0aCI6MjAuNDYwNjE4OTcyNzc4MzIsImhlaWdodCI6MS45MDUxNDg2MjUzNzM4NDAzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjcwLjUxMzAzMTAwNTg1OTM4LCJsZWZ0IjoxNy4zOTgyMTYyNDc1NTg1OTQsIndpZHRoIjo2Ni40MDM1NDkxOTQzMzU5NCwiaGVpZ2h0IjoxLjkwNTE1NzA4OTIzMzM5ODR9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NzIuMTM5MDQ1NzE1MzMyMDMsImxlZnQiOjE5LjA4ODM1NjAxODA2NjQwNiwid2lkdGgiOjY1Ljc4ODUyMDgxMjk4ODI4LCJoZWlnaHQiOjMuNTI4NDYwNTAyNjI0NTExN319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo3Ny4wMTcwOTc0NzMxNDQ1MywibGVmdCI6MTcuMzk4MjE2MjQ3NTU4NTk0LCJ3aWR0aCI6NjIuMDk0NTcwMTU5OTEyMTEsImhlaWdodCI6MS45MDUxNTcwODkyMzMzOTg0fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjc4LjY0MzExMjE4MjYxNzE5LCJsZWZ0IjoxOS4wODgzNTYwMTgwNjY0MDYsIndpZHRoIjozNi43MjUzMTEyNzkyOTY4NzUsImhlaWdodCI6MS45MDI0NDQyNDM0MzEwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjgxLjg5NTEzMzk3MjE2Nzk3LCJsZWZ0IjoxMy4yNDY4NTk1NTA0NzYwNzQsIndpZHRoIjo0MC4zOTEzOTU1Njg4NDc2NTYsImhlaWdodCI6MS45MDUxNTcwODkyMzMzOTg0fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjg1LjE0NzE2MzM5MTExMzI4LCJsZWZ0IjoxNS41ODM0NTg5MDA0NTE2Niwid2lkdGgiOjE3LjYwMjE3ODU3MzYwODQsImhlaWdodCI6MS45MDUxNTcwODkyMzMzOTg0fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjg4LjM5OTE5MjgxMDA1ODYsImxlZnQiOjE3LjM5ODIxNjI0NzU1ODU5NCwid2lkdGgiOjY1LjAwNzM0NzEwNjkzMzYsImhlaWdodCI6MS45MDUxNTcwODkyMzMzOTg0fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjkwLjAyNTIwNzUxOTUzMTI1LCJsZWZ0IjoxOS4wODgzNTYwMTgwNjY0MDYsIndpZHRoIjo2MS4wNTEwNTIwOTM1MDU4NiwiaGVpZ2h0IjoxLjkwMjQ0NDI0MzQzMTA5MTN9fSx7InBhZ2VOdW1iZXIiOjEzLCJiYm94Ijp7InRvcCI6MTAuMzUwNDE3MTM3MTQ1OTk2LCJsZWZ0IjoyMC44NDA4MDUwNTM3MTA5MzgsIndpZHRoIjo2Ni4yNzcwNTM4MzMwMDc4MSwiaGVpZ2h0IjozLjUyODQ1NTI1NzQxNTc3MTV9fSx7InBhZ2VOdW1iZXIiOjEzLCJiYm94Ijp7InRvcCI6MTUuNDk5NDY3ODQ5NzMxNDQ1LCJsZWZ0IjoxNy4zMzU5MDY5ODI0MjE4NzUsIndpZHRoIjoyMy41MTk2MTUxNzMzMzk4NDQsImhlaWdodCI6MS45MDUxNDg2MjUzNzM4NDAzfX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjE4Ljc1MTUwMTA4MzM3NDAyMywibGVmdCI6MTkuMTUwNjY1MjgzMjAzMTI1LCJ3aWR0aCI6NjQuNDM4NzY2NDc5NDkyMTksImhlaWdodCI6MS45MDUxNDg2MjUzNzM4NDAzfX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjIwLjM3NzUxNzcwMDE5NTMxMiwibGVmdCI6MjAuODQwODA1MDUzNzEwOTM4LCJ3aWR0aCI6NjEuMDc0NDI0NzQzNjUyMzQ0LCJoZWlnaHQiOjMuNTI4NDU0NTQyMTYwMDM0fX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjI1LjI1NTU2NzU1MDY1OTE4LCJsZWZ0IjoxOS4xNTA2NjUyODMyMDMxMjUsIndpZHRoIjo2OC4wMjM1NTk1NzAzMTI1LCJoZWlnaHQiOjEuOTA1MTQ4NjI1MzczODQwM319LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjoyNi44ODE1ODIyNjAxMzE4MzYsImxlZnQiOjIwLjg0MDgwNTA1MzcxMDkzOCwid2lkdGgiOjY2LjA3NjI5Mzk0NTMxMjUsImhlaWdodCI6My41Mjg0NzA5OTMwNDE5OTJ9fSx7InBhZ2VOdW1iZXIiOjEzLCJiYm94Ijp7InRvcCI6MzIuMDMwNjUxMDkyNTI5MywibGVmdCI6MTcuMzM1OTA2OTgyNDIxODc1LCJ3aWR0aCI6MjQuNjc3OTc2NjA4Mjc2MzY3LCJoZWlnaHQiOjEuOTA1MTQ4NjI1MzczODQwM319LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjozNS4yODI2ODQzMjYxNzE4NzUsImxlZnQiOjE5LjE1MDY2NTI4MzIwMzEyNSwid2lkdGgiOjYyLjQxOTc4ODM2MDU5NTcsImhlaWdodCI6MS45MDUxNDg2MjUzNzM4NDAzfX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjM2LjkwODY5OTAzNTY0NDUzLCJsZWZ0IjoyMC44NDA4MDUwNTM3MTA5MzgsIndpZHRoIjo2LjYzMzk5MzE0ODgwMzcxMSwiaGVpZ2h0IjoxLjkwMjQ0MDE5MDMxNTI0NjZ9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIxMyJ9fSx7Im9yaWdpbiI6NSwid29yayI6eyJ0eXBlIjo4LCJzdWJ0eXBlIjp7Im9uZW9mS2luZCI6ImFydGljbGVUeXBlIiwiYXJ0aWNsZVR5cGUiOjF9LCJjb250cmlidXRvcnMiOlt7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNaWNhaCBNLiIsImZhbWlseSI6IlN3YW5uIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFsaWNpYSIsImZhbWlseSI6IkNvcnTDqXMifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQWxleGFuZGVyIEwuIiwiZmFtaWx5IjoiRm9ycmVzdCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJOaWNob2xhcyBULiIsImZhbWlseSI6IkZyYW1zdGVkIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlN0ZXZlbiIsImZhbWlseSI6IlNhZHJvIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikdlb2ZmcmV5IiwiZmFtaWx5IjoiU2NobGFkb3cifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW5nZWxhIERlIiwiZmFtaWx5IjoiUGFsbWHigJBEb3cifX1dLCJpZHMiOnsiaXNzbiI6WyIxMDE1LTE2MjEiLCIxNDIwLTkwNTUiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMDcvczAwMDI3LTAyNC0wMTA0NS0yIiwib3BlbmFsZXgiOiJXNDM5MjcxMzM5OSIsImV4YSI6Imh0dHBzOi8vbGluay5zcHJpbmdlci5jb20vYXJ0aWNsZS8xMC4xMDA3L3MwMDAyNy0wMjQtMDEwNDUtMj9lcnJvcj1jb29raWVzX25vdF9zdXBwb3J0ZWQmY29kZT04M2NmOGQ4OC1jOWEwLTRjZGQtODI1Ni0xYTg1ZTZhYmRhYzMifSwidGl0bGUiOiJJbnRlcm5hbCBwaG9zcGhvcnVzIGxvYWRpbmcgYWx0ZXJzIG51dHJpZW50IGxpbWl0YXRpb24gYW5kIGNvbnRyaWJ1dGVzIHRvIGN5YW5vYmFjdGVyaWFsIGJsb29tcyBpbiBhIHBvbHltaWN0aWMgbGFrZSIsImFic3RyYWN0IjoiQWJzdHJhY3QgQ2xlYXIgTGFrZSwgYSBtZWRpdW0tc2l6ZWQgaHlwZXJldXRyb3BoaWMsIHBvbHltaWN0aWMgbGFrZSBpbiBub3J0aGVybiBDYWxpZm9ybmlhLCBoYXMgaGFkIHJlY3VycmluZyBoYXJtZnVsIGN5YW5vYmFjdGVyaWEgYmxvb21zIChIQ0JzKSBmb3Igb3ZlciBhIGNlbnR1cnkgZGVzcGl0ZSByZWR1Y3Rpb25zIGluIGV4dGVybmFsIHBob3NwaG9ydXMgKFApIGxvYWRpbmdzLiBJbnRlcm5hbCBQIGxvYWRpbmcgY2FuIGFsdGVyIG51dHJpZW50IGF2YWlsYWJpbGl0eSBhbmQgbGltaXRhdGlvbiBzdXBwb3J0aW5nIEhDQnMgYnV0IGlzIHJhcmVseSBxdWFudGlmaWVkIG9yIGNvbXBhcmVkIHdpdGggZXh0ZXJuYWwgbG9hZHMuIFdlIGhhdmUgcXVhbnRpZmllZCBleHRlcm5hbCBQIGxvYWRzIGZyb20gMjAxOSB0byAyMDIxIGZvciB0aGUgdGhyZWUgbWFpbiB0cmlidXRhcmllcyAoYWNjb3VudGluZyBmb3IgNDYlIG9mIHRoZSBmbG93KSBhbmQgaW50ZXJuYWwgbG9hZGluZ3MgdXNpbmcgdHdvIG1ldGhvZHM6IGEgUCBtYXNzIGJhbGFuY2UgYW5kIG1vZGVsZWQgcmVsZWFzZSByYXRlcyBvZiBzb2x1YmxlIHJlYWN0aXZlIHBob3NwaG9ydXMgZnJvbSBveGljIGFuZCBhbm94aWMgc2VkaW1lbnRzLiBJbiBhZGRpdGlvbiwgd2UgY29tYmluZWQgaGlnaC1mcmVxdWVuY3kgaW4gc2l0dSBtZWFzdXJlbWVudHMgb2Ygd2F0ZXIgdGVtcGVyYXR1cmUgYW5kIGRpc3NvbHZlZCBveHlnZW4sIGRpc2NyZXRlIGdyYWIgc2FtcGxpbmcgZm9yIG51dHJpZW50IGNoZW1pc3RyeSwgYW5kIHJlbW90ZSBzZW5zaW5nIHRvIGV4cGxvcmUgdGhlIHBvdGVudGlhbCBkcml2ZXJzIG9mIHRoZSBvYnNlcnZlZCB2YXJpYWJpbGl0eSBhbmQgcHJvdmlkZSBhIGNvbXByZWhlbnNpdmUgdmlldyBvZiB0aGUgc3BhdGlvdGVtcG9yYWwgZHluYW1pY3Mgb2YgSENCcy4gQnkgdW5kZXJzdGFuZGluZyB0aGUgcmVsYXRpdmUgY29udHJpYnV0aW9uIG9mIGV4dGVybmFsIGFuZCBpbnRlcm5hbCBudXRyaWVudCBsb2FkaW5ncyBhbmQgdGhlIHJlbGF0aW9uc2hpcCBiZXR3ZWVuIGVudmlyb25tZW50YWwgcGFyYW1ldGVycyBhbmQgSENCcywgaW50ZXJhbm51YWwgYmxvb20gdmFyaWFiaWxpdHkgY2FuIGJlIGJldHRlciBwcmVkaWN0ZWQuIENvbXBhcmF0aXZlIGVzdGltYXRlcyBvZiBleHRlcm5hbCBhbmQgaW50ZXJuYWwgcGhvc3Bob3J1cyBsb2FkaW5nIGluZGljYXRlIHRoYXQgaW50ZXJuYWwgc291cmNlcyBhY2NvdW50ZWQgZm9yIDcw4oCTOTUlIG9mIHRoZSB0b3RhbCBQIGlucHV0IGludG8gdGhlIHN5c3RlbSBkdXJpbmcgdGhlIHN0dWR5IHBlcmlvZC4gQ29udHJhcnkgdG8gb3RoZXIgbGFrZXMsIHRoZSBpbnRlbnNpdHkgb2YgdGhlIHN1bW1lciBibG9vbSBzZWFzb24gd2FzIGNvcnJlbGF0ZWQgdG8gdGhlIHRpbWluZyBhbmQgZHVyYXRpb24gb2YgYW5veGlhIHJhdGhlciB0aGFuIHRoZSBtYWduaXR1ZGUgb2Ygc3ByaW5nIHJ1bm9mZi4gSW50ZXJuYWxseSByZWxlYXNlZCBQIHNoaWZ0ZWQgdGhlIHN5c3RlbSBmcm9tIHBob3NwaG9ydXMgdG8gbml0cm9nZW4gbGltaXRhdGlvbiBkdXJpbmcgdGhlIHN1bW1lciwgcG90ZW50aWFsbHkgZmF2b3JpbmcgdGhlIHByb2xpZmVyYXRpb24gb2Ygbml0cm9nZW4tZml4aW5nIGN5YW5vYmFjdGVyaWEuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjEwMDcvczAwMDI3LTAyNC0wMTA0NS0yIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2xpbmsuc3ByaW5nZXIuY29tL2NvbnRlbnQvcGRmLzEwLjEwMDcvczAwMDI3LTAyNC0wMTA0NS0yLnBkZiIsImlzc3VlZERhdGUiOiIyMDI0LTAzLTEyIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMSIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiQXF1YXRpYyBTY2llbmNlcyIsImlzc3VlIjoyLCJ2b2x1bWUiOjg2LCJpbXBhY3RGYWN0b3IiOiIyLjg0In0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlNwcmluZ2VyIFNjaWVuY2UrQnVzaW5lc3MgTWVkaWEifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjc1LCJjaXRlZEJ5Q291bnQiOjIxfX0sInF1b3RlIjoiQ29udHJhcnkgdG8gb3RoZXIgbGFrZXMsIHRoZSBpbnRlbnNpdHkgb2YgdGhlIHN1bW1lciBibG9vbSBzZWFzb24gd2FzIGNvcnJlbGF0ZWQgdG8gdGhlIHRpbWluZyBhbmQgZHVyYXRpb24gb2YgYW5veGlhIHJhdGhlciB0aGFuIHRoZSBtYWduaXR1ZGUgb2Ygc3ByaW5nIHJ1bm9mZi4gSW50ZXJuYWxseSByZWxlYXNlZCBQIHNoaWZ0ZWQgdGhlIHN5c3RlbSBmcm9tIHBob3NwaG9ydXMgdG8gbml0cm9nZW4gbGltaXRhdGlvbiBkdXJpbmcgdGhlIHN1bW1lciwgcG90ZW50aWFsbHkgZmF2b3JpbmcgdGhlIHByb2xpZmVyYXRpb24gb2Ygbml0cm9nZW4tZml4aW5nIGN5YW5vYmFjdGVyaWEuIC0gW0NhbGNpdW0gYW5kIHBob3NwaGF0ZSBtZXRhYm9saWMgZGlzb3JkZXJzXShodHRwczovL2xpbmsuc3ByaW5nZXIuY29tL3N1YmplY3RzL2NhbGNpdW0tYW5kLXBob3NwaGF0ZS1tZXRhYm9saWMtZGlzb3JkZXJzKSBFdXRyb3BoaWNhdGlvbiBwb3NlcyBhIGdyYXZlIHRocmVhdCB0byBsZW50aWMgZWNvc3lzdGVtcyBnbG9iYWxseSwgcHJvbW90aW5nIHRoZSBwcm9saWZlcmF0aW9uIG9mIGhhcm1mdWwgY3lhbm9iYWN0ZXJpYSBibG9vbXMgKEhDQnMpIHRoYXQgcmVkdWNlIGFxdWF0aWMgYmlvZGl2ZXJzaXR5LCB5aWVsZGluZyBpbmNyZWFzZWQgd2F0ZXIgdHJlYXRtZW50IGNvc3RzLCBhbmQgdGhyZWF0ZW5pbmcgaHVtYW4gaGVhbHRoIHRocm91Z2ggYmxvb20tcHJvZHVjZWQgY3lhbm90b3hpbnMgKEFyw6FveiBldCBhbC4ifV0%3D. These diazotrophs exploit excess P via atmospheric N2 fixation, forming buoyant scums that dominate blooms (Jilbert et al., 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRvbSIsImZhbWlseSI6IkppbGJlcnQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUmFvdWzigJBNYXJpZSIsImZhbWlseSI6IkNvdXR1cmUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQnJpYW4iLCJmYW1pbHkiOiJIdXNlciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLYWxldmkiLCJmYW1pbHkiOiJTYWxvbmVuIn19XSwiaWRzIjp7Imlzc24iOlsiMDAxOC04MTU4IiwiMTU3My01MTE3Il0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDA3L3MxMDc1MC0wMjAtMDQ0NTcteCIsIm1hZyI6IjMxMDA0NTkwNTAiLCJvcGVuYWxleCI6IlczMTAwNDU5MDUwIn0sInRpdGxlIjoiUHJlZmFjZTogUmVzdG9yYXRpb24gb2YgZXV0cm9waGljIGxha2VzOiBjdXJyZW50IHByYWN0aWNlcyBhbmQgZnV0dXJlIGNoYWxsZW5nZXMiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAwNy9zMTA3NTAtMDIwLTA0NDU3LXgiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vbGluay5zcHJpbmdlci5jb20vY29udGVudC9wZGYvMTAuMTAwNy9zMTA3NTAtMDIwLTA0NDU3LXgucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjAtMTEtMTYiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA3IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJIeWRyb2Jpb2xvZ2lhIiwiaXNzdWUiOjIxLCJ2b2x1bWUiOjg0NywiaW1wYWN0RmFjdG9yIjoiMy4wMCJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjQzNDMsImxhc3RQYWdlIjo0MzU3fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlNwcmluZ2VyIFNjaWVuY2UrQnVzaW5lc3MgTWVkaWEifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjk1LCJjaXRlZEJ5Q291bnQiOjU4fX0sInF1b3RlIjoiU2ltaWxhciBkZWdyYWRhdGlvbiBvZiBmcmVzaHdhdGVyIGVjb3N5c3RlbXMgaXMgb2JzZXJ2ZWQgd29ybGR3aWRlLCB3aXRoIHRoZSB0aW1lbGluZSBvZiBodW1hbiBwcmVzc3VyZSBkaWN0YXRlZCBieSB0cmVuZHMgaW4gdXJiYW5pemF0aW9uIGFuZCBpbnRlbnNpdmUgYWdyaWN1bHR1cmUgaW4gZGlmZmVyZW50IGdlb2dyYXBoaWMgcmVnaW9ucyAoZS5nLiwgVmVubmVtbyBldCBhbC4sIDIwMDkpLiBUaGUgcHJpbmNpcGFsIGRyaXZlciBvZiBlY29zeXN0ZW0gc3RyZXNzIGluIG1vc3QgYXF1YXRpYyBzeXN0ZW1zIGlzIGV1dHJvcGhpY2F0aW9uLCB0aGUgZXhjZXNzIGdyb3d0aCBvZiBhbGdhZSBkcml2ZW4gYnkgYW50aHJvcG9nZW5pYyBpbnB1dHMgb2YgdGhlIG1hY3JvbnV0cmllbnRzIG5pdHJvZ2VuIChOKSBhbmQgcGhvc3Bob3J1cyAoUCkgKFNtaXRoLCAyMDAzOyBCaXJrIGV0IGFsLiwgMjAyMCkuIE51dHJpZW50cyBhcmUgZW1pdHRlZCBmcm9tIGJvdGggcG9pbnQgYW5kIGRpZmZ1c2Ugc291cmNlcyBpbiB0aGUgaHVtYW4gZW52aXJvbm1lbnQgYW5kIGVudGVyIGFxdWF0aWMgc3lzdGVtcyB2aWEgc3VyZmFjZSBydW5vZmYsIGFzIHdlbGwgYXMgZ3JvdW5kLXdhdGVyIGFuZCBhdG1vc3BoZXJpYyBkZXBvc2l0aW9uIChQYWVybCwgMTk5NykuIEV1dHJvcGhpY2F0aW9uIG9mdGVuIGxlYWRzIHRvIHRoZSBkb21pbmFuY2Ugb2YgcHJpbWFyeSBwcm9kdWN0aW9uIGJ5IGN5YW5vYmFjdGVyaWEsIGluY2x1ZGluZyB0b3hpYyBnZW5lcmEgc3VjaCBhcyBEb2xpY2hvc3Blcm11bSwgQXBoYW5pem9tZW5vbiwgYW5kIE1pY3JvY3lzdGlzIChIdWlzbWFuIGV0IGFsLiwgMjAxOCksIGFzIHdlbGwgYXMgb3h5Z2VuIGRlcGxldGlvbiBhbmQgaGFiaXRhdCBkZWdyYWRhdGlvbiBpbiBsYWtlcyAoQXJlbmQgZXQgYWwuLCAyMDExKS4gQmVjYXVzZSBsYWtlcyBwcm92aWRlIGEgcmFuZ2Ugb2YgY3J1Y2lhbCBlY29zeXN0ZW0gc2VydmljZXMgKFdpbHNvbiAmIENhcnBlbnRlciwgMTk5OSksIHRoZXJlIGlzIGEgZ3Jvd2luZyBuZWVkIGZvciBtYW5hZ2VtZW50IGFwcHJvYWNoZXMgdGhhdCByZXN0b3JlIHdhdGVyIHF1YWxpdHkgaW4gZnJlc2h3YXRlciBzeXN0ZW1zLiBXaXRoaW4gdGhlIGZyYW1ld29yayBvZiBVbml0ZWQgTmF0aW9ucyBTdXN0YWluYWJsZSBEZXZlbG9wLW1lbnQgR29hbCA2IChDbGVhbiBXYXRlciBhbmQgU2FuaXRhdGlvbiksIHNldmVyYWwgZGlzdGluY3QgdGFyZ2V0cyByZWxhdGUgdG8gdGhlIGltcHJvdmVtZW50IG9mIHdhdGVyIHF1YWxpdHkgYW5kIHByb3RlY3Rpb24gb2YgYXF1YXRpYyBlY29zeXN0ZW1zIGJ5IDIwMzAgKFVuaXRlZCBOYXRpb25zLCAyMDE1KS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyOC4yOTg4NjQzNjQ2MjQwMjMsImxlZnQiOjguNTQ5MjA1NzgwMDI5Mjk3LCJ3aWR0aCI6MzkuMzcxNDk0MjkzMjEyODksImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MjkuOTkxMTM0NjQzNTU0Njg4LCJsZWZ0Ijo4LjU0OTIwNTc4MDAyOTI5Nywid2lkdGgiOjM5LjM2NTExOTkzNDA4MjAzLCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMxLjY4MzQwNjgyOTgzMzk4NCwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNjk2NjMyMzg1MjUzOSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozMy4zNjc5NzMzMjc2MzY3MiwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNzU2NzkwMTYxMTMyOCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNS4wNjAyNDkzMjg2MTMyOCwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNzA5NjQwNTAyOTI5NywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNi43NTI1MTc3MDAxOTUzMSwibGVmdCI6OC41NDkyMDg2NDEwNTIyNDYsIndpZHRoIjozOS4zNzY5NjA3NTQzOTQ1MywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC40MzcwODgwMTI2OTUzMSwibGVmdCI6OC41NDkyMDg2NDEwNTIyNDYsIndpZHRoIjozOS4zNjY1Njk1MTkwNDI5NywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0MC4xMjkzNTYzODQyNzczNDQsImxlZnQiOjguNTQ5MjA4NjQxMDUyMjQ2LCJ3aWR0aCI6MzkuMzY0Mzc5ODgyODEyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0MS44MjE2MjQ3NTU4NTkzNzUsImxlZnQiOjguNTQ5MjA4NjQxMDUyMjQ2LCJ3aWR0aCI6MzkuMzY1NjczMDY1MTg1NTUsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDMuNTA2MTk1MDY4MzU5Mzc1LCJsZWZ0Ijo4LjU0OTM4OTgzOTE3MjM2Mywid2lkdGgiOjM5LjM3MDM5MTg0NTcwMzEyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0NS4xOTg0NjM0Mzk5NDE0MDYsImxlZnQiOjguNTQ5NTcyOTQ0NjQxMTEzLCJ3aWR0aCI6MzkuMzY1ODQ0NzI2NTYyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ni44OTA3MzU2MjYyMjA3LCJsZWZ0Ijo4LjU0OTU3Mjk0NDY0MTExMywid2lkdGgiOjM5LjM3Mjk1NTMyMjI2NTYyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0OC41NzUzMDIxMjQwMjM0NCwibGVmdCI6OC41NDk1NzI5NDQ2NDExMTMsIndpZHRoIjozOS4zNzI0MjEyNjQ2NDg0NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1MC4yNjc1NzgxMjUsImxlZnQiOjguNTQ5NTc2NzU5MzM4Mzc5LCJ3aWR0aCI6MzkuMzcxMTI0MjY3NTc4MTI1LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjUxLjk1OTg0MjY4MTg4NDc2NiwibGVmdCI6OC41NDk1NzY3NTkzMzgzNzksIndpZHRoIjozOS4zNjc4MzU5OTg1MzUxNTYsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTMuNTc0MTE5NTY3ODcxMDk0LCJsZWZ0Ijo4LjU0OTU3Njc1OTMzODM3OSwid2lkdGgiOjM5LjM3MDc3NzEzMDEyNjk1LCJoZWlnaHQiOjEuMzM2ODk1NTg1MDYwMTE5Nn19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjU1LjI2NjM5MTc1NDE1MDM5LCJsZWZ0Ijo4LjU0OTc2MDgxODQ4MTQ0NSwid2lkdGgiOjM5LjM2NjU4MDk2MzEzNDc2NiwiaGVpZ2h0IjoxLjMzNjg5NTU4NTA2MDExOTZ9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1Ny4wMjg5NTM1NTIyNDYwOTQsImxlZnQiOjguNTQ5OTQ0ODc3NjI0NTEyLCJ3aWR0aCI6MzkuMzYyMDI2MjE0NTk5NjEsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTguNzEzNTIzODY0NzQ2MDk0LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjkuMjk5OTgyMDcwOTIyODUyLCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjYwLjQwNTc5MjIzNjMyODEyNSwibGVmdCI6MTAuNjIyNDU1NTk2OTIzODI4LCJ3aWR0aCI6MzcuMjg3ODY4NDk5NzU1ODYsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NjIuMDk4MDYwNjA3OTEwMTU2LCJsZWZ0Ijo4LjU0OTk0MzkyMzk1MDE5NSwid2lkdGgiOjM5LjM3MzMyNTM0NzkwMDM5LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjYzLjc4MjYzMDkyMDQxMDE1NiwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzMxMjY5ODM2NDI1OCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2NS40NzQ5MDY5MjEzODY3MiwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNjUxMTk5MzQwODIwMywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2Ny4xNjcxNzUyOTI5Njg3NSwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNjk0ODAxMzMwNTY2NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2OC43ODE0NDgzNjQyNTc4MSwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzczMzg0MDk0MjM4MywiaGVpZ2h0IjoxLjMzNjg5MTUzMTk0NDI3NX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjcwLjU0NDAxMzk3NzA1MDc4LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjM5LjM2OTMwODQ3MTY3OTY5LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjcyLjIzNjI4OTk3ODAyNzM0LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjM5LjM3MjAzMjE2NTUyNzM0NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo3My45MjA4NDUwMzE3MzgyOCwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzEzMDczNzMwNDY4NzUsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNDM0NCJ9fV0%3D. Conversely, N-overenrichment from urea promotes non-fixing toxin-producers such as Microcystis and Planktothrix, indicators of sustained external N loading essential for CyanoHABs (Paerl, 2018)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhhbnMgVy4iLCJmYW1pbHkiOiJQYWVybCJ9fV0sImlkcyI6eyJpc3NuIjpbXSwiaXNibiI6W10sImRvaSI6IjEwLjE3NjE1L3luNTUtYjQzOCIsIm9wZW5hbGV4IjoiVzQzMDcwNjM3MzkifSwidGl0bGUiOiJNaXRpZ2F0aW5nIHRveGljIHBsYW5rdG9uaWMgY3lhbm9iYWN0ZXJpYWwgYmxvb21zIGluIGFxdWF0aWMgZWNvc3lzdGVtcyBmYWNpbmcgaW5jcmVhc2luZyBhbnRocm9wb2dlbmljIGFuZCBjbGltYXRpYyBwcmVzc3VyZXMiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTc2MTUveW41NS1iNDM4IiwicmVzb3VyY2VVcmwiOiJodHRwczovL2Nkci5saWIudW5jLmVkdS9kb3dubG9hZHMvd2gyNDcyOTBnIiwiaXNzdWVkRGF0ZSI6IjIwMTgtMDEtMDEiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA2IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJDYXJvbGluYSBEaWdpdGFsIFJlcG9zaXRvcnkgKFVuaXZlcnNpdHkgb2YgTm9ydGggQ2Fyb2xpbmEgYXQgQ2hhcGVsIEhpbGwpIiwiaW1wYWN0RmFjdG9yIjoiMC4xMCJ9LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJVbml2ZXJzaXR5IG9mIE5vcnRoIENhcm9saW5hIGF0IENoYXBlbCBIaWxsIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjowLCJjaXRlZEJ5Q291bnQiOjB9fSwicXVvdGUiOiJUaGlzIGxlYWRzICB0byBjb250aW51aW5nIGRlbWFuZCBmb3IgTiBzdXBwbGllcyB0byBzdXN0YWluIHByaW1hcnkgcHJvZHVjdGlvbiBhbmQgcHJvbW90ZSBldXRyb3BoaWNhdGlvbiAgWzM4LDM5XSwgKDIpIGEgbGVuZ3RoeSBsZWdhY3kgb2YgUCBsb2FkaW5nIHRoYXQgaXMgc3RvcmVkIGluIHNlZGltZW50cywgd2hpY2ggaXMgcmVhZGlseSByZWN5Y2xlZCBhbmQgIHN1cHBvcnRzIGV1dHJvcGhpY2F0aW9uLCAoMykgTjIgZml4YXRpb24sIHdoaWxlIHByb3ZpZGluZyDigJxuZXfigJ0gTiwgZmFpbHMgdG8gbWVldCBlY29zeXN0ZW0tc2NhbGUgTiAgcmVxdWlyZW1lbnRzIFs0MCw0MV0uIEZyb20gYSBudXRyaWVudCBtYW5hZ2VtZW50IHBlcnNwZWN0aXZlLCB0aGlzIG1lYW5zIHRoYXQgY29udGludWVkIGV4dGVybmFsICBsb2FkaW5nIG9mIE4gcGxheXMgYSBjcml0aWNhbCByb2xlIGluIGFjY2VsZXJhdGluZyBldXRyb3BoaWNhdGlvbiBhbmQgcHJvbW90aW5nIGFsZ2FsIGJsb29tcyBbMzZdLiAgT25lIHRyb3VibGluZyBpbmRpY2F0b3Igb2YgTi1kcml2ZW4gZXV0cm9waGljYXRpb24gYW5kIEN5YW5vSEFCcyBpbiBmcmVzaHdhdGVyIHN5c3RlbXMgaXMgIHRoZSBnZW9ncmFwaGljIGV4cGFuc2lvbiBhbmQgaW5jcmVhc2UgZnJlcXVlbmN5IG9mIHRoZSBub24tTjIgZml4aW5nIHRveGluLXByb2R1Y2luZyBnZW51cyAgTWljcm9jeXN0aXMgWzQyXSwgZXNwZWNpYWxseSBpbiBsYXJnZSBsYWtlcyAoZS5nLiwgTGFrZSBFcmllLCBDYW5hZGEtVVNBLCBMYWtlIEthc3VtYWd1cmEsIEphcGFuLCBMYWtlICBPa2VlY2hvYmVlLCBGbG9yaWRhLCBVU0EsIExha2UgVGFpaHUsIENoaW5hKS4gVGhpcyBnZW51cywgYXMgd2VsbCBhcyBhbm90aGVyIHBsYW5rdG9uaWMgbm9uLU4yICBmaXhpbmcgQ3lhbm9IQUIgUGxhbmt0b3Rocml4LCBhbmQgc2V2ZXJhbCBiZW50aGljIGFuYWxvZ3VlIGJsb29tIHRheGEgKGUuZy4sIG5vbi1OMiBmaXhpbmcgTHluZ2J5YSAgYW5kIE9zY2lsbGF0b3JpYSBzcHAuKSwgc2VydmUgYXMgaW5kaWNhdG9ycyBvZiBOLW92ZXItZW5yaWNobWVudCBbMzYsNDJdLiBUaGUgcHJvbGlmZXJhdGlvbiBvZiB0aGVzZSBub24tIEZpZ3VyZSA1LiBHbG9iYWwgTiBhbmQgUC1iYXNlZCBmZXJ0aWxpemVyIHVzZSBpbiByZWxhdGlvbiB0byB0aGUgd29ybGTigJlzIGh1bWFuIHBvcHVsYXRpb24uICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NzUuNDY1MTI2MDM3NTk3NjYsImxlZnQiOjIwLjA1ODI5NDI5NjI2NDY1LCJ3aWR0aCI6NjQuMDM1Njk3OTM3MDExNzIsImhlaWdodCI6MS4wODg0MjM2MDk3MzM1ODE1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NzYuOTA4MzAyMzA3MTI4OSwibGVmdCI6MTYuODQ2OTUyNDM4MzU0NDkyLCJ3aWR0aCI6NjcuMjM4ODYxMDgzOTg0MzgsImhlaWdodCI6MS4wODg0MjM2MDk3MzM1ODE1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NzguMzUxNDc4NTc2NjYwMTYsImxlZnQiOjE2Ljg0Njk1MjQzODM1NDQ5Miwid2lkdGgiOjY3LjI0NzA3MDMxMjUsImhlaWdodCI6MS4wODg0MjM2MDk3MzM1ODE1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NzkuNzk0NjU0ODQ2MTkxNCwibGVmdCI6MTYuODQ2OTUyNDM4MzU0NDkyLCJ3aWR0aCI6NjcuMjUwMDIyODg4MTgzNiwiaGVpZ2h0IjoxLjA4ODUxMDYzMjUxNDk1MzZ9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo4MS4yMzc5MjI2Njg0NTcwMywibGVmdCI6MTYuODQ3MDQwMTc2MzkxNiwid2lkdGgiOjY3LjI0MDA1MTI2OTUzMTI1LCJoZWlnaHQiOjEuMDg4NDIzNjA5NzMzNTgxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjgyLjY4MTA5ODkzNzk4ODI4LCJsZWZ0IjoxNi44NDcwNDAxNzYzOTE2LCJ3aWR0aCI6NjQuNzA4OTE1NzEwNDQ5MjIsImhlaWdodCI6MS4wODg0MjM2MDk3MzM1ODE1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6ODQuMTI0MjgyODM2OTE0MDYsImxlZnQiOjIwLjA1ODM4Mzk0MTY1MDM5LCJ3aWR0aCI6NjQuMDI3MjA2NDIwODk4NDQsImhlaWdodCI6MS4wODg0MjM2MDk3MzM1ODE1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6ODUuNTYxMDI3NTI2ODU1NDcsImxlZnQiOjE2Ljg0NzAzNjM2MTY5NDMzNiwid2lkdGgiOjY3LjIzMzM2NzkxOTkyMTg4LCJoZWlnaHQiOjEuMDg4NDU5ODQ5MzU3NjA1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6ODcuMDA0MjQxOTQzMzU5MzgsImxlZnQiOjE2Ljg0NzAwNzc1MTQ2NDg0NCwid2lkdGgiOjY3LjI0ODIyMjM1MTA3NDIyLCJoZWlnaHQiOjEuMDg4NDIzNjA5NzMzNTgxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjg4LjQ0NzQwMjk1NDEwMTU2LCJsZWZ0IjoxNi44NDcwMDc3NTE0NjQ4NDQsIndpZHRoIjo2Ny4wODMxNTI3NzA5OTYxLCJoZWlnaHQiOjEuMDg4NDIzNjA5NzMzNTgxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjg5Ljg5MDYwMjExMTgxNjQsImxlZnQiOjE2Ljg0NzAwNzc1MTQ2NDg0NCwid2lkdGgiOjY3LjIzNDg5Mzc5ODgyODEyLCJoZWlnaHQiOjEuMDg4NDIzNjA5NzMzNTgxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjkxLjMzMzc3ODM4MTM0NzY2LCJsZWZ0IjoxNi44NDY5ODQ4NjMyODEyNSwid2lkdGgiOjY2Ljg2MjQxMTQ5OTAyMzQ0LCJoZWlnaHQiOjEuMDg4NDIzNjA5NzMzNTgxNX19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjU2LjgxODE2NDgyNTQzOTQ1LCJsZWZ0IjoxNi40Mjg1MTQ0ODA1OTA4Miwid2lkdGgiOjYuMzU2NDE0MzE4MDg0NzE3LCJoZWlnaHQiOjEuMzE2Mzc4NDc0MjM1NTM0N319LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjU2LjgwOTYzODk3NzA1MDc4LCJsZWZ0IjoyNC40OTQzNTIzNDA2OTgyNDIsIndpZHRoIjo1OS4zNDEzMDA5NjQzNTU0NywiaGVpZ2h0IjoxLjMwNTczMjEzMTAwNDMzMzV9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI2In19XQ%3D%3D. In Indian contexts, inefficient application only 30-40% nutrient uptake by crops exacerbates leaching, worsened by minimal buffer zones around lakes. Monsoon torrents mobilize soluble reactive P and ammonium, rapidly bioavailable for algal uptake, converting inorganic nutrients to biomass within days (Sitote & Gebremedhine, 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllhYnNyYSBNZWxhayIsImZhbWlseSI6IlNpdG90ZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNaWNoYWVsIEdpcmltYXkiLCJmYW1pbHkiOiJHZWJyZW1lZGhpbmUifX1dLCJpZHMiOnsiaXNzbiI6W10sImlzYm4iOltdLCJkb2kiOiIxMC4yMDk0NC9wcmVwcmludHMyMDI0MDkuMDcwNC52MSIsIm9wZW5hbGV4IjoiVzQ0MDI0NjU0MTQiLCJleGEiOiJodHRwczovL3d3dy5wcmVwcmludHMub3JnL21hbnVzY3JpcHQvMjAyNDA5LjA3MDQifSwidGl0bGUiOiJDb21wcmVoZW5zaXZlIFJldmlldyBvZiBFdXRyb3BoaWNhdGlvbiBpbiBGcmVzaHdhdGVyIEVjb3N5c3RlbXM6IENhdXNlcywgRWZmZWN0cywgQXNzZXNzbWVudCwgYW5kIE1hbmFnZW1lbnQgU3RyYXRlZ2llcyIsImFic3RyYWN0IjoiVGhpcyByZXZpZXcgZXhhbWluZXMgZXV0cm9waGljYXRpb24gaW4gZnJlc2h3YXRlciBlY29zeXN0ZW1zLCBhIHdpZGVzcHJlYWQgZW52aXJvbm1lbnRhbCBpc3N1ZSBzdGVtbWluZyBmcm9tIGV4Y2Vzc2l2ZSBudXRyaWVudCBpbnB1dCwgbGFyZ2VseSBkdWUgdG8gaHVtYW4gYWN0aXZpdGllcy4gVGhlIHN0dWR5IGV4cGxvcmVzIHRoZSBvcmlnaW5zLCBjb25zZXF1ZW5jZXMsIGV2YWx1YXRpb24gdGVjaG5pcXVlcywgYW5kIGNvbnRyb2wgbWV0aG9kcyBmb3IgZXV0cm9waGljYXRpb24uIFByaW1hcnkgY2F1c2VzIGluY2x1ZGUgYWdyaWN1bHR1cmFsIGRpc2NoYXJnZSwgdXJiYW4gZGV2ZWxvcG1lbnQsIGFuZCBjbGltYXRlIHNoaWZ0cy4gVGhlIGVmZmVjdHMgYXJlIGZhci1yZWFjaGluZywgaW1wYWN0aW5nIGJpb2RpdmVyc2l0eSwgd2F0ZXIgcXVhbGl0eSwgYW5kIHNvY2lvLWVjb25vbWljIGZhY3RvcnMuIFRoZSBhc3Nlc3NtZW50IHV0aWxpemVzIGEgY29tYmluYXRpb24gb2YgY2hlbWljYWwsIHBoeXNpY2FsLCBhbmQgYmlvbG9naWNhbCBtYXJrZXJzLCB3aXRoIGN1dHRpbmctZWRnZSB0ZWNobm9sb2dpZXMgbGlrZSByZW1vdGUgc2Vuc2luZyBpbXByb3ZpbmcgbW9uaXRvcmluZyBlZmZvcnRzLiBNaXRpZ2F0aW9uIHN0cmF0ZWdpZXMgYXJlIGRpdmVyc2UsIGluY29ycG9yYXRpbmcgcHJldmVudGF0aXZlIGFjdGlvbnMgKHN1Y2ggYXMgbnV0cmllbnQgbGltaXRhdGlvbiBhbmQgbGFuZCBtYW5hZ2VtZW50KSwgY29ycmVjdGl2ZSBtZWFzdXJlcyAoaW5jbHVkaW5nIGJpby1tYW5pcHVsYXRpb24gYW5kIGNoZW1pY2FsIGFwcGxpY2F0aW9ucyksIGFuZCByZWd1bGF0b3J5IGluaXRpYXRpdmVzIChsaWtlIHdhdGVyIHF1YWxpdHkgZ3VpZGVsaW5lcyBhbmQgbnV0cmllbnQgZXhjaGFuZ2Ugc2NoZW1lcykuIFdoaWxlIGNhc2Ugc3R1ZGllcyBkZW1vbnN0cmF0ZSBzdWNjZXNzZnVsIGludGVydmVudGlvbnMsIHRoZXkgYWxzbyByZXZlYWwgb25nb2luZyBjaGFsbGVuZ2VzLCBwYXJ0aWN1bGFybHkgZWNvc3lzdGVtIHVucHJlZGljdGFiaWxpdHkgYW5kIGNsaW1hdGUgY2hhbmdlIGluZmx1ZW5jZXMuIFRoZSByZXNlYXJjaCBoaWdobGlnaHRzIHRoZSBwcm9taXNlIG9mIGVtZXJnaW5nIHRlY2hub2xvZ2llcywgbm90YWJseSBtYWNoaW5lIGxlYXJuaW5nLCBpbiBlbmhhbmNpbmcgbWFuYWdlbWVudCBwcmFjdGljZXMuIEtub3dsZWRnZSBnYXBzIHBlcnNpc3QsIGVzcGVjaWFsbHkgcmVnYXJkaW5nIGV1dHJvcGhpY2F0aW9uLWJpb2RpdmVyc2l0eSBkeW5hbWljcyBpbiB0cm9waWNhbCBlbnZpcm9ubWVudHMgYW5kIHRoZSBzb2Npby1lY29ub21pYyByYW1pZmljYXRpb25zIG9mIGludGVydmVudGlvbnMuIFRoZSBzdHVkeSBlbXBoYXNpemVzIHRoZSBpbXBvcnRhbmNlIG9mIGZsZXhpYmxlLCBjb21wcmVoZW5zaXZlIGFwcHJvYWNoZXMgdGhhdCBibGVuZCBzY2llbnRpZmljIGluc2lnaHRzIHdpdGggcHJhY3RpY2FsIHNvbHV0aW9ucyBhbmQgc3Ryb25nIHBvbGljeSBzdHJ1Y3R1cmVzIHRvIHRhY2tsZSB0aGlzIGludHJpY2F0ZSBlbnZpcm9ubWVudGFsIGNoYWxsZW5nZSBpbiBhbiBlcmEgb2YgY2xpbWF0ZSBjaGFuZ2UuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjIwOTQ0L3ByZXByaW50czIwMjQwOS4wNzA0LnYxIiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5wcmVwcmludHMub3JnL2Zyb250ZW5kL21hbnVzY3JpcHQvZTA4ZTViNTY1MzNlMmFhZDc0N2ZkOTQzNWU2N2NhNzMvZG93bmxvYWRfcHViIiwiaXNzdWVkRGF0ZSI6IjIwMjQtMDktMTAiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTEyIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJQcmVwcmludHMub3JnIiwiaW1wYWN0RmFjdG9yIjoiMS4xNSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MCwiY2l0ZWRCeUNvdW50Ijo1fX0sInF1b3RlIjoiVGhlIHN0dWR5IGVtcGhhc2l6ZXMgdGhlIGltcG9ydGFuY2Ugb2YgZmxleGlibGUsIGNvbXByZWhlbnNpdmUgYXBwcm9hY2hlcyB0aGF0IGJsZW5kIHNjaWVudGlmaWMgaW5zaWdodHMgd2l0aCBwcmFjdGljYWwgc29sdXRpb25zIGFuZCBzdHJvbmcgcG9saWN5IHN0cnVjdHVyZXMgdG8gdGFja2xlIHRoaXMgaW50cmljYXRlIGVudmlyb25tZW50YWwgY2hhbGxlbmdlIGluIGFuIGVyYSBvZiBjbGltYXRlIGNoYW5nZS4gRXV0cm9waGljYXRpb24sIGNhdXNlZCBieSBleGNlc3NpdmUgbnV0cmllbnQgZW5yaWNobWVudCBpbiB3YXRlciBib2RpZXMsIGlzIGEgc2lnbmlmaWNhbnQgZW52aXJvbm1lbnRhbCBjaGFsbGVuZ2UgZm9yIGZyZXNod2F0ZXIgZWNvc3lzdGVtcyBvbiBhIGdsb2JhbCBzY2FsZS4gUHJpbWFyaWx5IGRyaXZlbiBieSBuaXRyb2dlbiBhbmQgcGhvc3Bob3J1cyBpbnB1dHMsIHRoaXMgcHJvY2VzcyByZXN1bHRzIGluIHRoZSBvdmVycHJvZHVjdGlvbiBvZiBhbGdhZSBhbmQgYXF1YXRpYyBwbGFudHMsIGxlYWRpbmcgdG8gYSBjYXNjYWRlIG9mIGVjb2xvZ2ljYWwgZGlzdHVyYmFuY2VzLCBpbmNsdWRpbmcgb3h5Z2VuIGRlcGxldGlvbiAoaHlwb3hpYSksIGxvc3Mgb2YgYmlvZGl2ZXJzaXR5LCBhbmQgc2V2ZXJlIHdhdGVyIHF1YWxpdHkgZGVncmFkYXRpb24gKEtha2FkZSBldCBhbC4sIDIwMjE7IE1lZXJob2ZmIGV0IGFsLiwgMjAyMjsgV3VydHNiYXVnaCBldCBhbC4sIDIwMTkpLiJ9XQ%3D%3D.

Beyond fertilizers, pesticides like organophosphates and herbicides (e.g., glyphosate, atrazine) compound the crisis by entering waterways at 1-10 μg/L concentrations, selectively eliminating grazers like Daphnia while sparing resilient cyanobacteria (Çelekli̇, 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFidXplciIsImZhbWlseSI6IsOHZWxla2xpzIcifX1dLCJpZHMiOnsiaXNzbiI6W10sImlzYm4iOltdLCJkb2kiOiIxMC4yMTIwMy9ycy4zLnJzLTUwNzgxNzYvdjEiLCJvcGVuYWxleCI6Ilc0NDA0NTI5NDE4In0sInRpdGxlIjoiVGhlIGZpcnN0IGVjby1hc3Nlc3NtZW50IG9mIHBoeXRvcGxhbmt0b24gYXNzZW1ibGFnZXMgYXNzb2NpYXRlZCB3aXRoIGVudmlyb25tZW50YWwgY29uZGl0aW9ucyBvZiBkaWZmZXJlbnQgbGFrZXMgaW4gdGhlIFdlc3Rlcm4gQmxhY2sgU2VhIGJhc2luIG9mIFTDvHJraXllIiwiYWJzdHJhY3QiOiI8dGl0bGU%2BQWJzdHJhY3Q8L3RpdGxlPiBMYWtlcyBwbGF5IGEgdml0YWwgcm9sZSBpbiBzdXBwb3J0aW5nIGJpb2RpdmVyc2l0eSwgcHJvdmlkaW5nIHdhdGVyIHJlc291cmNlcywgcmVndWxhdGluZyBjbGltYXRlLCBjeWNsaW5nIG51dHJpZW50cywgYW5kIG9mZmVyaW5nIHJlY3JlYXRpb25hbCBvcHBvcnR1bml0aWVzLiBEZXNwaXRlIHRoZWlyIGltcG9ydGFuY2UgZm9yIGVudmlyb25tZW50YWwgaGVhbHRoIGFuZCBodW1hbiB3ZWxsLWJlaW5nLCBsYWtlcyBmYWNlIHNpZ25pZmljYW50IHByZXNzdXJlcyBpbiB0aGUgQW50aHJvcG9jZW5lIGVyYS4gVGhlIHByZXNlbnQgd29yayBzZWVrcyB0byBhc3Nlc3MgdGhlIHNwZWNpZXMtZW52aXJvbm1lbnQgaW50ZXJhY3Rpb25zIGFuZCB0aGUgZWNvbG9naWNhbCBzdGF0dXMgb2Ygc2l4IGxha2VzIGluIHRoZSBXZXN0ZXJuIEJsYWNrIFNlYSBiYXNpbiBvZiBUw7xya2l5ZSB1dGlsaXppbmcgcGh5dG9wbGFua3RvbiBtZXRyaWNzIGR1cmluZyB3ZXQgYW5kIGRyeSBwZXJpb2RzLiBDYW5vbmljYWwgY29ycmVzcG9uZGVuY2UgYW5hbHlzaXMgcmV2ZWFsZWQgYSBzaWduaWZpY2FudCBjb3JyZWxhdGlvbiBlcXVhbCB0byA5OC41JSBiZXR3ZWVuIHBoeXRvcGxhbmt0b24gc3BlY2llcyBhbmQgZW52aXJvbm1lbnRhbCBzdHJlc3NvcnMgZHVyaW5nIHR3byBoeWRyb2xvZ2ljYWwgcGVyaW9kcy4gRWxlY3RyaWNhbCBjb25kdWN0aXZpdHkgKEVDKSwgcEgsIHRvdGFsIG9yZ2FuaWMgY2FyYm9uIChUT0MpLCBhbmQgdGVtcGVyYXR1cmUgd2VyZSB0aGUgbW9zdCBpbmZsdWVudGlhbCBlbnZpcm9ubWVudGFsIGZhY3RvcnMgYWZmZWN0aW5nIHBoeXRvcGxhbmt0b24gZGlzdHJpYnV0aW9uIGluIGxha2VzIChwPTAuMDAyKS4gTGFrZSBTYXLEsWt1bSwgYSBicmFja2lzaCBoYWJpdGF0LCB3YXMgdW5kZXIgaGlnaCBFQyBhbmQgcEggcHJlc3N1cmUgYW5kIGlzIGNoYXJhY3Rlcml6ZWQgYnkgcG9sbHV0aW9uLXRvbGVyYW50IHNwZWNpZXMuIExha2UgWWVuacOnYcSfYSBhc3NvY2lhdGVkIHdpdGggVE9DIGlzIGNoYXJhY3Rlcml6ZWQgYnkgc29tZSBzcGVjaWVzLCBzdWNoIGFzIDxpdGFsaWM%2BQW5hYmFlbm9wc2lzPC9pdGFsaWM%2BIDxpdGFsaWM%2BbWlsbGVyaTwvaXRhbGljPiwgPGl0YWxpYz5DaHJvb2NvY2N1czwvaXRhbGljPiA8aXRhbGljPnR1cmdpZHVzPC9pdGFsaWM%2BLDxpdGFsaWM%2BIFBzZXVkb3NjaHJvZWRlcmlhPC9pdGFsaWM%2BIDxpdGFsaWM%2Bcm9idXN0YTwvaXRhbGljPiwgPGl0YWxpYz5BcGhhbm9jYXBzYTwvaXRhbGljPiBzcC4sIDxpdGFsaWM%2BTWVyaXNtb3BlZGlhPC9pdGFsaWM%2BIDxpdGFsaWM%2BZ2xhdWNhPC9pdGFsaWM%2BLCA8aXRhbGljPk1pY3JhY3Rpbml1bTwvaXRhbGljPiA8aXRhbGljPnF1YWRyaXNldHVtPC9pdGFsaWM%2BLCBhbmQ8aXRhbGljPiBNaWNyb2N5c3RpczwvaXRhbGljPiA8aXRhbGljPmFlcnVnaW5vc2E8L2l0YWxpYz4uIExha2UgQWJhbnQgaXMgbG9jYXRlZCBvbiB0aGUgb3Bwb3NpdGUgc2lkZSBvZiBFQywgVE9DLCBhbmQgdGVtcGVyYXR1cmUsIHdoaWNoIHdhcyBhc3NvY2lhdGVkIHdpdGggc29tZSBzcGVjaWVzIHN1Y2ggYXMgPGl0YWxpYz5DeW1iZWxsYSBhZmZpbmlzPC9pdGFsaWM%2BLCA8aXRhbGljPkFjaG5hbnRoaWRpdW08L2l0YWxpYz4gPGl0YWxpYz5taW51dGlzc2ltdW08L2l0YWxpYz4sIDxpdGFsaWM%2BRW5jeW9uZW1hPC9pdGFsaWM%2BIDxpdGFsaWM%2BbWludXR1bTwvaXRhbGljPiwgPGl0YWxpYz5FLjwvaXRhbGljPiA8aXRhbGljPnNpbGVzaWFjdW08L2l0YWxpYz4sIGFuZDxpdGFsaWM%2BIERpbm9icnlvbjwvaXRhbGljPiA8aXRhbGljPmRpdmVyZ2VuczwvaXRhbGljPi4gUmVzdWx0cyBvZiB0aGUgcGh5dG8tYXNzZXNzbWVudCBkaXNwbGF5ZWQgdGhhdCB0aGUgZWNvbG9naWNhbCBzdGF0dXMgKEVTKSBvZiB0aGUgc2FtcGxpbmcgc3RhdGlvbnMgZHVyaW5nIHRoZSByYWlueSBhbmQgZHJ5IHBlcmlvZHMgdmFyaWVkIGZyb20gYmFkIHRvIGhpZ2guIFRoZSBtb2RpZmllZCBwaHl0b3BsYW5rdG9uIHRyb3BoaWMgaW5kZXggKE1QVEkpIGV4aGliaXRlZCB0aGF0IGEgbW9kZXJhdGUgRVMgd2FzIGZvdW5kIGluIExha2VzIFNhcsSxa3VtIGFuZCBZZW5pw6dhxJ9hLCB3aGlsZSBvdGhlcnMgaGFkIGEgZ29vZCBFUy4gVGhlIHByZXNlbnQgc3R1ZHkgY29u76yBcm1lZCB0aGF0IHBoeXRvcGxhbmt0b24gY29tbXVuaXRpZXMgYXJlIHN0cm9uZ2x5IGxpbmtlZCB0byB0aGUgZWNvbG9naWNhbCBzdGF0dXMgb2YgbGFrZXMgaW4gdGhlIFdlc3Rlcm4gQmxhY2sgU2VhIGJhc2luLCB3aGljaCBjb3VsZCBiZSBhc3Nlc3NlZCB1c2luZyB0aGUgTVBUSS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjEyMDMvcnMuMy5ycy01MDc4MTc2L3YxIiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5yZXNlYXJjaHNxdWFyZS5jb20vYXJ0aWNsZS9ycy01MDc4MTc2L2xhdGVzdC5wZGYiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMS0xOSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDgiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IlJlc2VhcmNoIFNxdWFyZSAoUmVzZWFyY2ggU3F1YXJlKSJ9LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJSZXNlYXJjaCBTcXVhcmUgKFVuaXRlZCBTdGF0ZXMpIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo1MiwiY2l0ZWRCeUNvdW50IjowfX0sInF1b3RlIjoiRGVzcGl0ZSB0aGVpciBlY29sb2dpY2FsIHNpZ25p77%2B9Y2FuY2UsIHRoZXNlIHdhdGVyIGJvZGllcyBhcmUgaW5jcmVhc2luZ2x5IHRocmVhdGVuZWQgYnkgYW50aHJvcG9nZW5pYyBhY3Rpdml0aWVzIChlLmcuLCB1cmJhbml6YXRpb24sIGFncmljdWx0dXJhbCBsYW5kIHVzZSwgd2FzdGV3YXRlciBkaXNjaGFyZ2UsIHJlZWQgY3V0dGluZywgc2FuZCByZW1vdmFsLCBldGMuKSBhbmQgY2xpbWF0ZSBjaGFuZ2UgKEjDpGRlciBldCBhbC4gMjAyMDsgw4dlbGVrbGkgZXQgYWwuIDIwMjQpLiBOdXRyaWVudCBlbnJpY2htZW50IGluIGxlbnRpYyBlY29zeXN0ZW1zIGltcGFjdHMgbm90IG9ubHkgdGhlIHBoeXNpY29jaGVtaWNhbCBmYWN0b3JzIG9mIGxha2VzIGJ1dCBhbHNvIG5lZ2F0aXZlbHkgYWZmZWN0cyBhcXVhdGljIGxpZmUsIGNhdXNpbmcgaGFybWZ1bCBhbGdhbCBibG9vbXMsIGV4Y2Vzc2l2ZSBncm93dGggb2YgbnVpc2FuY2UgbWFjcm9waHl0ZXMsIGFuZCBpbmNyZWFzZWQg77%2B9c2ggbW9ydGFsaXR5LCBlc3BlY2lhbGx5IHNlbnNpdGl2ZSBzcGVjaWVzIChTdXJlc2ggZXQgYWwuIDIwMjMpLiBJdCBhbHNvIGRpc3J1cHRzIHRoZSBoeWRyb21vcnBob2xvZ3kgYW5kIG92ZXJhbGwgZnVuY3Rpb25pbmcgb2YgdGhlc2UgZWNvc3lzdGVtcy4gRGVzcGl0ZSBnbG9iYWwgd2F0ZXIgc2hvcnRhZ2VzLCB0aGUgZ3Jvd2luZyBkZWdyYWRhdGlvbiBvZiBlY29zeXN0ZW1zIGFuZCB0aGUgb3ZlcnVzZSBvZiBmcmVzaHdhdGVyIHJlc291cmNlcyBoYXZlIHNoaWZ0ZWQgYXR0ZW50aW9uIGJhY2sgdG8gZnJlc2h3YXRlciBlY29zeXN0ZW1zIHRvIGJldHRlciBtYW5hZ2UgYW5kIHByZXNlcnZlIHdhdGVyIHJlc291cmNlcyBnbG9iYWxseSAow4dlbGVrbGkgZXQgYWwuIDIwMjE7IFN1cmVzaCBldCBhbC4gMjAyMykuIEFzc2Vzc2luZyB0aGUgZWNvbG9naWNhbCBzdGF0dXMgb2YgbGFrZXMgdGhyb3VnaCBwaHl0b3BsYW5rdG9uIG1ldHJpY3MgaXMgYSBjcnVjaWFsIGNvbXBvbmVudCBvZiBlbnZpcm9ubWVudGFsIG1vbml0b3JpbmcgYW5kIG1hbmFnZW1lbnQsIGFzIGl0IGFsaWducyB3aXRoIHRoZSBndWlkZWxpbmVzIHNldCBieSB0aGUgV2F0ZXIgRnJhbWV3b3JrIERpcmVjdGl2ZSAoRGlyZWN0aXZlIDIwMDApLiBQaHl0b3BsYW5rdG9uLCBhcyBwcmltYXJ5IHByb2R1Y2VycywgYXJlIHNlbnNpdGl2ZSBpbmRpY2F0b3JzIG9mIGNoYW5nZXMgaW4gd2F0ZXIgcXVhbGl0eSBhbmQgZWNvc3lzdGVtIGhlYWx0aCAoUmV5bm9sZHMgZXQgYWwuIDIwMDI7IMOHZWxla2xpIGV0IGFsLiAyMDIzYSkuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NjYuOTY0MjI1NzY5MDQyOTcsImxlZnQiOjQuNTM0MzEzNjc4NzQxNDU1LCJ3aWR0aCI6OTAuNjE0NzYxMzUyNTM5MDYsImhlaWdodCI6MS43NzU1Mjk2MjMwMzE2MTYyfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NjkuMjM2OTA3OTU4OTg0MzgsImxlZnQiOjQuNTM0MzEzNjc4NzQxNDU1LCJ3aWR0aCI6OTAuMDEwMzA3MzEyMDExNzIsImhlaWdodCI6MS43NzU1Mjk2MjMwMzE2MTYyfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NzEuNTA5NTkwMTQ4OTI1NzgsImxlZnQiOjQuNTM0MzEzNjc4NzQxNDU1LCJ3aWR0aCI6ODUuODEyMTc5NTY1NDI5NjksImhlaWdodCI6MS43NzU1Mjk2MjMwMzE2MTYyfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NzMuNzgyMjcyMzM4ODY3MTksImxlZnQiOjQuNTM0MzEzNjc4NzQxNDU1LCJ3aWR0aCI6ODYuMDcwMzEyNSwiaGVpZ2h0IjoxLjc3NTUyOTYyMzAzMTYxNjJ9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo3Ni4wNTQ5NTQ1Mjg4MDg2LCJsZWZ0Ijo0LjUzNDMxMzY3ODc0MTQ1NSwid2lkdGgiOjg5LjYyMzY2NDg1NTk1NzAzLCJoZWlnaHQiOjEuNzc1NTI5NjIzMDMxNjE2Mn19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjc4LjMyNzYyOTA4OTM1NTQ3LCJsZWZ0Ijo0LjUzNDMxMzY3ODc0MTQ1NSwid2lkdGgiOjg5Ljg1OTc1NjQ2OTcyNjU2LCJoZWlnaHQiOjEuNzc1NTI5NjIzMDMxNjE2Mn19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjgwLjYwMDMxODkwODY5MTQsImxlZnQiOjQuNTM0MzEzNjc4NzQxNDU1LCJ3aWR0aCI6ODkuNjQwODM4NjIzMDQ2ODgsImhlaWdodCI6MS43NzU1Mjk2MjMwMzE2MTYyfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6ODIuODcyOTkzNDY5MjM4MjgsImxlZnQiOjQuNTM0MzEzNjc4NzQxNDU1LCJ3aWR0aCI6OTAuODM2NjM5NDA0Mjk2ODgsImhlaWdodCI6NC4wNDgxOTQ4ODUyNTM5MDZ9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo4Ny40MTgzNDI1OTAzMzIwMywibGVmdCI6NC41MzQzMTM2Nzg3NDE0NTUsIndpZHRoIjozMy43OTA5MzU1MTYzNTc0MiwiaGVpZ2h0IjoxLjc3NTUyOTYyMzAzMTYxNjJ9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo5MS4yMDYxNDYyNDAyMzQzOCwibGVmdCI6NC41MzQzMTM2Nzg3NDE0NTUsIndpZHRoIjo4NC42NzUzNzY4OTIwODk4NCwiaGVpZ2h0IjoxLjc3NTUyOTYyMzAzMTYxNjJ9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo5My40Nzg4MjA4MDA3ODEyNSwibGVmdCI6NC41MzQzMTM2Nzg3NDE0NTUsIndpZHRoIjo4OS45ODM4NDg1NzE3NzczNCwiaGVpZ2h0IjoxLjc3NTUyOTYyMzAzMTYxNjJ9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo0Ljc0OTY0MzgwMjY0MjgyMiwibGVmdCI6NC41MzQzMTM2Nzg3NDE0NTUsIndpZHRoIjo4Ny45NTc4NjI4NTQwMDM5LCJoZWlnaHQiOjEuNzc1NTMwNTc2NzA1OTMyNn19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjcuMDIyMzA5MzAzMjgzNjkxLCJsZWZ0Ijo0LjUzNDMxMzY3ODc0MTQ1NSwid2lkdGgiOjkwLjY2NjUxMTUzNTY0NDUzLCJoZWlnaHQiOjEuNzc1NTMwNTc2NzA1OTMyNn19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjIifX1d. These chemicals induce physiological stress in algae, paradoxically boosting toxin production, and inhibit macrophyte growth, reducing natural nutrient sinks (Shahbaz et al., 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1kIiwiZmFtaWx5IjoiU2hhaGJheiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBaGJhciIsImZhbWlseSI6IkFsYW0ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTW9oYW1tYWQgTWFzcm9vciIsImZhbWlseSI6IlphZmFyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1vaGFtbWVkIEFhc2lmIiwiZmFtaWx5IjoiU3VsYWltYW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW51cG1hIiwiZmFtaWx5IjoiS3VtYXJpIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkcuIEsuIiwiZmFtaWx5IjoiU2hhcm1hIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNoYWhsYSIsImZhbWlseSI6Illhc21pbiJ9fV0sImlkcyI6eyJpc3NuIjpbIjEyMTUtMTYxMCIsIjIwNjEtOTU4OCJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMjQ3OC9vcmh1LTIwMjMtMDAxNiIsIm9wZW5hbGV4IjoiVzQzODk1MTAwMzcifSwidGl0bGUiOiJDb21wYXJpc29uIG9mIGF2aWFuIGRpdmVyc2l0eSBiZXR3ZWVuIG1hbmFnZWQgYW5kIHVubWFuYWdlZCB3ZXRsYW5kcyBpbiBQYXRuYSwgQmloYXIsIEluZGlhIiwiYWJzdHJhY3QiOiJBYnN0cmFjdCBXZSBjb21wYXJlZCB0aGUgYmlyZCBkaXZlcnNpdHkgb2YgYSBtYW5hZ2VkIChSYWpkaGFuaSBKYWxhc2hheSkgYW5kIGFuIHVubWFuYWdlZCB3ZXRsYW5kIChNdXN0YWZhcHVyKSBpbiBQYXRuYSwgQmloYXIsIEluZGlhLiBXZSByZWNvcmRlZCBiaXJkIHNwZWNpZXMgYnkgdXNpbmcgcG9pbnQgY291bnRzIGFuZCBmaXhlZC1yb3V0ZSBtb25pdG9yaW5nLiBBdCBSYWpkaGFuaSBKYWxhc2hheSwgYSB0b3RhbCBvZiA3MyBzcGVjaWVzIHdlcmUgcmVjb3JkZWQsIG9mIHdoaWNoIDE1IHNwZWNpZXMgd2VyZSB3aW50ZXIgdmlzaXRvcnMuIFR3byBzcGVjaWVzIHdlcmUgaW4gdGhlIE5lYXItdGhyZWF0ZW5lZCBjYXRlZ29yeTogRmVycnVnaW5vdXMgRHVjayAoQXl0aHlhIG55cm9jYSkgYW5kIEFsZXhhbmRyaW5lIFBhcmFrZWV0IChQc2l0dGFjdWxhIGV1cGF0cmlhKSAsIHdoaWxlIHRoZSBvdGhlcnMgd2VyZSBvZiBsZWFzdCBjb25jZXJuLiBBdCBNdXN0YWZhcHVyIHdldGxhbmQsIDY3IHNwZWNpZXMgd2VyZSByZWNvcmRlZCB3aXRoIDExIHNwZWNpZXMgYXMgd2ludGVyIHZpc2l0b3JzLiBTaW1wc29u4oCZcyBzcGVjaWVzIGRpdmVyc2l0eSBpbmRleCB3YXMgMC43OCBhdCBSYWpkaGFuaSBKYWxhc2hheSwgYW5kIDAuODEgYXQgTXVzdGFmYXB1ciB3ZXRsYW5kLiBUaGUgbnVtYmVyIG9mIHNwZWNpZXMgb2Ygd2FkZXJzIHdhcyAxNyBhdCBNdXN0YWZhcHVyIHdldGxhbmQgYW5kIDExIGF0IFJhamRoYW5pIEphbGFzaGF5LiBUaGUgdG90YWwgbnVtYmVyIG9mIExlc3NlciBXaGlzdGxpbmctZHVjayAoRGVuZHJvY3lnbmEgamF2YW5pY2EpICwgYSByZXNpZGVudCBiaXJkIHdhcyBhbG1vc3QgZG91YmxlICg1MjYpIGF0IE11c3RhZmFwdXIgd2V0bGFuZCB0aGFuIHRoYXQgYXQgUmFqZGhhbmkgSmFsYXNoYXkgKDIzNCkuIFRoZSBuaXRyYXRlIGxldmVsIHdhcyB0d2ljZSBoaWdoZXIgYW5kIHRoZSBwaG9zcGhhdGUgbGV2ZWwgd2FzIGZvdXIgdGltZXMgaGlnaGVyIGF0IE11c3RhZmFwdXIgd2V0bGFuZCB0aGFuIHRob3NlIGF0IFJhamRoYW5pIEphbGFzaGF5LiBUaGUgZGlmZmVyZW5jZSBpbiBzcGVjaWVzIG51bWJlciBhbmQgY29tcG9zaXRpb24gYmV0d2VlbiB0aGUgdHdvIHdldGxhbmRzIG1heSBiZSBhdHRyaWJ1dGVkIHRvIHRoZSBldXRyb3BoaWNhdGlvbiByZXN1bHRpbmcgZnJvbSBoaWdoIG5pdHJhdGUgYW5kIHBob3NwaGF0ZSBsZXZlbHMgYXQgTXVzdGFmYXB1ciB3ZXRsYW5kLiBVbmZvcnR1bmF0ZWx5LCB0aGUgTXVzdGFmYXB1ciB3ZXRsYW5kIGlzIHVuZGVyIHRocmVhdCBkdWUgdG8gaHVtYW4gaW5mbHVlbmNlcy4gVGhlIHByb3RlY3Rpb24gYW5kIHN1c3RhaW5hYmxlIG1hbmFnZW1lbnQgb2YgbmF0dXJhbCB3ZXRsYW5kcyBpcyByZXF1aXJlZCBmb3Igc2F2aW5nIHRoZSBiaW9kaXZlcnNpdHkgb2YgdGhlIGFyZWEuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjI0Nzgvb3JodS0yMDIzLTAwMTYiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vc2NpZW5kby5jb20vcGRmLzEwLjI0Nzgvb3JodS0yMDIzLTAwMTYiLCJpc3N1ZWREYXRlIjoiMjAyMy0xMi0wMSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6Ik9ybmlzIEh1bmdhcmljYSIsImlzc3VlIjoyLCJ2b2x1bWUiOjMxLCJpbXBhY3RGYWN0b3IiOiIwLjkyIn0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTMsImxhc3RQYWdlIjoyOH19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJIdW5nYXJpYW4gT3JuaXRob2xvZ2ljYWwgYW5kIE5hdHVyZSBDb25zZXJ2YXRpb24gU29jaWV0eSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MjcsImNpdGVkQnlDb3VudCI6MX19LCJxdW90ZSI6Ikhvd2V2ZXIsIGhpZ2ggbGV2ZWxzIG9mIGV1dHJvcGhpY2F0aW9uIG1heSBhbHNvIHRyaWdnZXIgdGhlIHByb2xpZmVyYXRpb24gb2YgZ3JlZW4gYWxnYWUgYW5kICBkZWNsaW5lIG9mIG1hY3JvcGh5dGVzIGxlYWRpbmcgdG8gYSBoaWdoLXR1cmJpZGl0eSBzdGF0ZSBvZiB3YXRlciwgaW4gd2hpY2ggZmV3IHNwZWNpZXMgY2FuICBzdXJ2aXZlIChTY2hlZmZlciBldCBhbC4gMTk5MykuIFRvIGFkZHJlc3MgdGhlIGV1dHJvcGhpY2F0aW9uIGFuZCBleGNlc3MgbnV0cmllbnQgaXNzdWVzIGluICBhcXVhdGljIGVjb3N5c3RlbXMsIHBoeXRvcmVtZWRpYXRpb24gb2ZmZXJzIGEgcHJvbWlzaW5nIHNvbHV0aW9uIChMaXUgZXQgYWwuIDIwMjEpLiBBcXVhdGljICBwbGFudHMgbGlrZSBkdWNrd2VlZHMsIHdhdGVyIGxldHR1Y2UgKFBpc3RpYSBzdHJhdGlvdGVzKSwgYW5kIHdhdGVyIGxpbGllcyBoYXZlIGRlbW9uc3RyYXRlZCAgdGhlaXIgYWJpbGl0eSB0byBjb21iYXQgZXV0cm9waGljYXRpb24gYnkgYWJzb3JiaW5nIGFuZCBhc3NpbWlsYXRpbmcgZXhjZXNzIHBob3NwaG9ydXMgYW5kICBuaXRyb2dlbiAoTmFoYXIgJiBIb3F1ZSAyMDIxKS4gSG93ZXZlciwgdGhlc2UgcGxhbnRzIGNhbiBzb21ldGltZXMgZ3JvdyB1bmNvbnRyb2xsYWJseSwgIHBvdGVudGlhbGx5IGNvdmVyaW5nIHRoZSB3YXRlciBzdXJmYWNlcy4gVG8gYXZvaWQgc3VjaCBzY2VuYXJpb3MsIHJlZ3VsYXIgbW9uaXRvcmluZyBpcyBlc3NlbnRpYWwgKExpdSBldCBhbC4gMjAyMSkuIFByb21vdGluZyBwdWJsaWMgYXdhcmVuZXNzIGlzIHRoZSBjb3JuZXJzdG9uZSBmb3IgbW9iaWxpemluZyBzdXBwb3J0IGFuZCBlbnN1cmluZyBlZmZlY3RpdmUgZXV0cm9waGljYXRpb24gbWFuYWdlbWVudC4gQ29udHJvbGxlZCBlcmFkaWNhdGlvbiBvZiBleGNlc3MgIGZsb3JhIHdoZW4gbmVjZXNzYXJ5IGlzIHZpdGFsIHRvIHByZXZlbnQgb3Zlcmdyb3d0aC4gVGhpcyBpbnRlZ3JhdGVkIGFwcHJvYWNoIGNhbiBlZmZlY3RpdmVseSAgcmVkdWNlIGV1dHJvcGhpY2F0aW9uIGluIHRoZSB3YXRlciBib2RpZXMgKFdhbmcgZXQgYWwuIDIwMjIpLiBFYWNoIHNwZWNpZXMgb2Ygd2V0bGFuZC1kZXBlbmRlbnQgYmlyZCBoYXMgYSB1bmlxdWUgYW5kIGNvbXBsZXggc2V0IG9mIG5lZWRzIGZvciB3ZXRsYW5kIGhhYml0YXRzIHRoYXQgbWFrZXMgaXQgZGlmZmljdWx0IHRvIGdlbmVyYWxpemUgYWJvdXQgaG93IGxvc3Mgb3IgZGVncmFkYXRpb24gb2Ygd2V0bGFuZHMgYWZmZWN0cyBiaXJkIHBvcHVsYXRpb25zLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo3OS4yOTU2MzkwMzgwODU5NCwibGVmdCI6MTAuMzAzMDA2MTcyMTgwMTc2LCJ3aWR0aCI6NzkuOTUyNzI4MjcxNDg0MzgsImhlaWdodCI6MS43MDkxOTc4Nzg4Mzc1ODU0fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjgxLjIxMjUzOTY3Mjg1MTU2LCJsZWZ0IjoxMC4zMDMwMDYxNzIxODAxNzYsIndpZHRoIjo3OS45NTkyODE5MjEzODY3MiwiaGVpZ2h0IjoxLjcwOTE5Nzg3ODgzNzU4NTR9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6ODMuMTI5NDQ3OTM3MDExNzIsImxlZnQiOjEwLjMwMzAwNjE3MjE4MDE3Niwid2lkdGgiOjc5Ljk0MjcyNjEzNTI1MzksImhlaWdodCI6MS43MDkxOTc4Nzg4Mzc1ODU0fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjg1LjA0NjM0ODU3MTc3NzM0LCJsZWZ0IjoxMC4zMDMwMDgwNzk1Mjg4MDksIndpZHRoIjo3OS45Mzk0NTMxMjUsImhlaWdodCI6MS43MDkxOTc4Nzg4Mzc1ODU0fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjg2Ljk2MzI1NjgzNTkzNzUsImxlZnQiOjEwLjMwMzAxMDk0MDU1MTc1OCwid2lkdGgiOjc5Ljk0MTg0ODc1NDg4MjgxLCJoZWlnaHQiOjEuNzA5MTk3ODc4ODM3NTg1NH19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo4OC44ODAxNjUxMDAwOTc2NiwibGVmdCI6MTAuMzAzMDEzODAxNTc0NzA3LCJ3aWR0aCI6NzkuOTM3NSwiaGVpZ2h0IjoxLjcwOTE5Nzg3ODgzNzU4NTR9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6OTAuNzk3MDY1NzM0ODYzMjgsImxlZnQiOjEwLjMwMzAxMzgwMTU3NDcwNywid2lkdGgiOjc5LjkzOTY1MTQ4OTI1NzgxLCJoZWlnaHQiOjEuNzA5MTk3ODc4ODM3NTg1NH19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo5Mi43MTM5NzM5OTkwMjM0NCwibGVmdCI6MTAuMzAzMDEzODAxNTc0NzA3LCJ3aWR0aCI6NzkuOTUwMzI1MDEyMjA3MDMsImhlaWdodCI6MS43MDkxOTc4Nzg4Mzc1ODU0fX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjEyLjIwNzk5MDY0NjM2MjMwNSwibGVmdCI6MTAuMzAzMDI0MjkxOTkyMTg4LCJ3aWR0aCI6NzkuOTUyNTE0NjQ4NDM3NSwiaGVpZ2h0Ijo3LjQ1OTkyMjc5MDUyNzM0NH19LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjoxOS44NzU2MjU2MTAzNTE1NjIsImxlZnQiOjEyLjEyMTMyNDUzOTE4NDU3LCJ3aWR0aCI6NzguMTE5MjI0NTQ4MzM5ODQsImhlaWdodCI6MS43MDkxOTc4Nzg4Mzc1ODU0fX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjIxLjc5MjUzMzg3NDUxMTcyLCJsZWZ0IjoxMC4zMDMwMjkwNjAzNjM3Nywid2lkdGgiOjc5Ljk0OTI0OTI2NzU3ODEyLCJoZWlnaHQiOjEuNzA5MTk3ODc4ODM3NTg1NH19LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjoyMy43MDk0MzY0MTY2MjU5NzcsImxlZnQiOjEwLjMwMzAyOTA2MDM2Mzc3LCJ3aWR0aCI6NzkuOTUyMjcwNTA3ODEyNSwiaGVpZ2h0IjoxLjcwOTE5Nzg3ODgzNzU4NTR9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIyNSJ9fV0%3D. Soil erosion during monsoons carries particle-bound P (70-90% of TP transport), depositing it in lake sediments where anoxic conditions later release it via mineralization (Haxthausen, 2021; Paerl, 2018)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkthcmwgUmFzbXVzIEF1Z3VzdCIsImZhbWlseSI6IkhheHRoYXVzZW4ifX1dLCJpZHMiOnsiaXNzbiI6W10sImlzYm4iOltdLCJtYWciOiIzMTc3MTEzMjA3Iiwib3BlbmFsZXgiOiJXMzE3NzExMzIwNyJ9LCJ0aXRsZSI6IlNlZGltZW50IG1pY3JvYmlhbCBmdWVsIGNlbGxzIGZvciBsYWtlIHJlc3RvcmF0aW9uIiwiYWJzdHJhY3QiOiJFdXRyb3BoaWNhdGlvbiBvZiBsYWtlcyBpcyB1YmlxdWl0b3VzLCBhcyAzMC00MCAlIG9mIHRoZSB3b3JsZOKAmXMgbGFrZXMgYXJlIGV1dHJvcGhpYywgYW5kIGNvbmRpdGlvbnMgY29udGludW91c2x5IGRldGVyaW9yYXRpbmcuIEV1dHJvcGhpY2F0aW9uIGlzIGEgY29uZGl0aW9uIG9mIG92ZXJmZXJ0aWxpemF0aW9uLCB3aGVyZSBhbGwgcmVxdWlyZW1lbnRzIGZvciByYXBpZCBncm93dGggb2YgYWxnYWUgYXJlIG1ldCwgcmVzdWx0aW5nIGluIGFsZ2FsIGJsb29tcy4gVGhpcyBjYXVzZXMgYSBzZWxmLXN1c3RhaW5lZCwgdmljaW91cyBjeWNsZSB3aGVyZSBwb29yIGVjb2xvZ2ljYWwgY29uZGl0aW9ucyBhcmUgbWFpbnRhaW5lZCB1bnRpbCBudXRyaWVudHMgYXJlIGxvc3QgZnJvbSB0aGUgbGFrZSBvdmVyIG1hbnkgeWVhcnMuIExha2UgcmVzdG9yYXRpb24gc2Vla3MgdG8gYnJlYWsgdGhpcyB2aWNpb3VzIGN5Y2xlLiBJbmFkZXF1YXRlIGF2YWlsYWJpbGl0eSBvZiBhbnkgc2luZ2xlIG51dHJpZW50IGluIHdpbGwgbGltaXQgYWxnYWwgZ3Jvd3RoLCB0eXBpY2FsbHkgcGhvc3Bob3J1cyBpcyB0YXJnZXRlZC4gVGhlIHNjb3BlIG9mIHRoaXMgUGhEIHRoZXNpcyB3YXMgdG8gZGV2ZWxvcCwgdGVzdCBhbmQgb3B0aW1pemUgc2VkaW1lbnQgbWljcm9iaWFsIGZ1ZWwgY2VsbHMgKFNNRkMpIGFzIGEgbm92ZWwgbWV0aG9kIGZvciBsYWtlIHJlc3RvcmF0aW9uLiBUaGUgdGhlc2lzIHNwYW5zIHRoZSB0d28gbWFpbiB0b3BpY3Mgb2YgU01GQ3MgYW5kIGxha2UgcmVzdG9yYXRpb24gdGVjaG5pcXVlcy4gTGFrZSByZXN0b3JhdGlvbiB0ZWNobmlxdWVzIGFyZSByZXZpZXdlZCwgcHJvdmlkaW5nIHVuZGVyc3RhbmRpbmcgb2YgdGhlIHVuZGVybHlpbmcgbWVjaGFuaXNtcy4gU01GQ3MgYXJlIGFwcHJvYWNoZWQgZnJvbSBhbiBlbmdpbmVlcmluZyBwZXJzcGVjdGl2ZSBhbmQgYSBmb2N1cyBvbiBsYWtlIHJlbWVkaWF0aW9uLiBUaGUgc3BlY2lmaWMgb2JqZWN0aXZlcyB3ZXJlIHRvIHRlc3QgYSBsb3ctY29zdCBlbGVjdHJvZGUgbWF0ZXJpYWwgZm9yIGl0cyBkdXJhYmlsaXR5LCBwcmFjdGljYWxpdHksIGFuZCBwZXJmb3JtYW5jZS4gUGVyZm9ybWFuY2Ugd2FzIGFzc2Vzc2VkIGFzIHRoZSBhYmlsaXR5IHRvIGxpbWl0IHBob3NwaG9ydXMgcmVsZWFzZSBmcm9tIHNlZGltZW50cywgaW5oaWJpdCBtZXRoYW5lIHJlbGVhc2UsIGFuZCB0aGUgYWRkZWQgYmVuZWZpdHMgb2YgZGVncmFkaW5nIHBvbGx1dGFudHMsIGFuZCBnZW5lcmF0aW5nIGVsZWN0cmljaXR5LiBQcm9vZiBvZiBjb25jZXB0IHdhcyBzaG93biBpbiBsYWJvcmF0b3J5IHJlYWN0b3JzICgxIEwpLCB0aGUgZWZmZWN0IG9mIHVwc2NhbGluZyB0byAyMDAgTCBpbiBuYXR1cmFsIGVudmlyb25tZW50IHdhcyBzdHVkaWVkIGluIG91dGRvb3IgbWVzb2Nvc21zLCBhbmQgdGhlbiB0ZXN0ZWQgYXQgcGlsb3Qgc2NhbGUgaW4gYSBldXRyb3BoaWMgZnJlc2h3YXRlciBsYWtlLiBQaWxvdCBpbnN0YWxsYXRpb25zIHdlcmUgdGhlIGxhcmdlc3QgaW4gdGhlIHdvcmxkIG9mIHRoZWlyIGtpbmQgYW5kIHdlcmUgb3BlcmF0ZWQgZm9yIDIgeWVhcnMuIFN0YWlubGVzcyBzdGVlbCB3YXMgc2VsZWN0ZWQgZm9yIHRlc3RpbmcgYXMgZWxlY3Ryb2RlIG1hdGVyaWFsIGR1ZSB0byBpdHMgbG93IGNvc3QsIGR1cmFiaWxpdHksIGFuZCBhdmFpbGFiaWxpdHkuIFBhaXJzIG9mIGVsZWN0cm9kZXMgd2VyZSBlbWJlZGRlZCBpbiB0aGUgc2VkaW1lbnQgKHRoaXMgaXMgY2FsbGVkIHRoZSBhbm9kZSkgYW5kIHN1c3BlbmRlZCBpbiB0aGUgb3Zlcmx5aW5nIHdhdGVyICh0aGlzIGlzIGNhbGxlZCB0aGUgY2F0aG9kZSkuIFRoZSB0d28gZWxlY3Ryb2RlcyB3ZXJlIGNvbm5lY3RlZCBhY3Jvc3MgYW4gZXh0ZXJuYWwgcmVzaXN0b3Igd2l0aCBlbGVjdHJpY2FsIHdpcmVzLiBNaWNyb2JlcyBpbiB0aGUgc2VkaW1lbnQgdXNlIHRoZSBhbm9kZSBwYWlyIGFzIGVsZWN0cm9uIGFjY2VwdG9yLCBlbmFibGluZyByZXNwaXJhdGlvbiBpbiB0aGUgYW5veGljIHNlZGltZW50LiBFbGVjdHJvbnMgdHJhdmVsIGZyb20gdGhlIHNlZGltZW50IHZpYSB0aGUgZWxlY3RyaWNhbCBjb25uZWN0b3IgdG8gdGhlIG92ZXJseWluZyBjYXRob2RlIHdoZXJlIG94eWdlbiBpbiB0aGUgd2F0ZXIgaXMgdGhlIHRlcm1pbmFsIGVsZWN0cm9uIGFjY2VwdG9yIGNvbXBsZXRpbmcgdGhlIGNpcmN1aXQuIEFzIGFuIGFkZGVkIGJlbmVmaXQsIGRlZ3JhZGF0aW9uIG9mIHBvbGx1dGFudHMgdXNpbmcgU01GQyB3YXMgdGVzdGVkIGluIG1hcmluZSBoYXJib3VyIHNlZGltZW50cywgYnV0IG5vIGNvbmNsdXNpdmUgcmVzdWx0cyBvYnRhaW5lZC4gTGFib3JhdG9yeSByZWFjdG9ycyB3ZXJlIG9wZXJhdGVkIGZvciBtb3JlIHRoYW4gMjAwIGRheXMgdXNpbmcgcGV0cm9sZXVtIGFzIHNvbGUgY2FyYm9uIHNvdXJjZSwgY29uZmlybWluZyBvaWwgZGVncmFkYXRpb24gYnkgU01GQ3MuIEEgcmVtb3ZhbCByYXRlIG9mIDAuOSBnIHBldHJvbGV1bSBtLTMgZC0xIHJlYWN0b3Igdm9sdW1lIHdhcyBlc3RpbWF0ZWQgZnJvbSBtZWFzdXJlZCBlbGVjdHJpY2FsIG91dHB1dC4gVGhpcyB3YXMgZG9uZSBieSBhc3N1bXB0aW9uIG9mIHRoZSBjaGVtaWNhbCByZWFjdGlvbiBvY2N1cnJpbmcsIGZyb20gdGhpcyB0aGUgbnVtYmVyIG9mIGVsZWN0cm9ucyB0cmFuc2ZlcnJlZCBmcm9tIG9pbCB0byB3YXRlciB3YXMga25vd24gYW5kIGNvdWxkIGJlIGNvcnJlbGF0ZWQgdG8gdGhlIG1lYXN1cmVkIGZsb3cgb2YgZWxlY3Ryb25zIChjdXJyZW50KSB0aHJvdWdoIHRoZSBleHRlcm5hbCBjaXJjdWl0LiBTdGFpbmxlc3Mgc3RlZWwgZWxlY3Ryb2RlcyB3ZXJlIG9wZXJhdGVkIGluIGZyZXNod2F0ZXIgbGFrZSBlbnZpcm9ubWVudHMgZm9yIG1vcmUgdGhhbiA0IHllYXJzIHdpdGggbWluaW1hbCBkYW1hZ2UgdG8gdGhlIG1hdGVyaWFsLiBUaGUgZWxlY3Ryb2RlIGlzIHRoZSBtYWluIG1hdGVyaWFsIGNvc3QgaW4gc3VjaCBzeXN0ZW1zLiBIb3dldmVyLCBBSVNJMzE2IHN0YWlubGVzcyBzdGVlbCBzaW1pbGFyIHRvIHdoYXQgd2FzIHVzZWQgaW4gdGhpcyB0aGVzaXMgd2FzIGF0IHRoZSB0aW1lIG9mIHdyaXRpbmcgYXZhaWxhYmxlIGF0IGJ1bGsgY29zdCBvZiAwLjA5MSDigqwgbS0yLCBjb3JyZXNwb25kaW5nIHRvIDkxMCDigqwgYWNyZS0xLCBzaWduaWZpY2FudGx5IGxvd2VyIHRoYW4gb3RoZXIgY29tbW9uIG1hdGVyaWFscy4gQ2F0aG9kZXMgYXQgbGFyZ2Ugc2NhbGUgY291bGQgYmUgbW92ZWQgaG9yaXpvbnRhbGx5IGFuZCB2ZXJ0aWNhbGx5IHdpdGggbWlub3IgaW1wYWN0IHRvIHBlcmZvcm1hbmNlLiBQaWxvdCBzY2FsZSBTTUZDcyBnZW5lcmF0ZWQgMC4zODMgbVcgbS0yIHN1c3RhaW5lZCBhbmQgMjcuNCBtVyBtLTIgcGVhayBvdXRwdXQuIFRoZSBzeXN0ZW0gd2FzIGxpbWl0ZWQgYnkgdGhlIGNhdGhvZGUgc3VyZmFjZSBhcmVhLCBtZWFuaW5nIG11bHRpLWxheWVyIGNhdGhvZGVzIHNob3VsZCBiZSB0ZXN0ZWQgaW4gdGhlIGZ1dHVyZSBpZiBlbGVjdHJpY2FsIHBvd2VyIHByb2R1Y3Rpb24gaXMgdGhlIGdvYWwuIFRoZSBjYXRob2RlIHNob3VsZCBiZSBwbGFjZWQgYXQgYSBkZXB0aCB0aGF0IGVuc3VyZXMgaGlnaCBkaXNzb2x2ZWQgb3h5Z2VuIGNvbmNlbnRyYXRpb25zIHRvIGF2b2lkIHRoZSBzeXN0ZW0gdG8gYmUgbGltaXRlZC4gSG93ZXZlciwgdG9vIGhpZ2ggZWxlY3RyaWNpdHkgZ2VuZXJhdGlvbiB3YXMgZm91bmQgY291bnRlcnByb2R1Y3RpdmUgZm9yIHJlbWVkaWF0aW9uIHB1cnBvc2VzIGJlY2F1c2UgdGhpcyB3b3VsZCBpbmNyZWFzZSBzZWRpbWVudCB0dXJub3ZlciByYXRlLCByZWxlYXNpbmcgcG9sbHV0YW50cyBzdG9yZWQgaW4gdGhlIHNlZGltZW50cy4gUCBiaW5kaW5nIHdhcyBub3QgZm91bmQgdG8gcmVxdWlyZSBoaWdoIHR1cm5vdmVyIG9mIHNlZGltZW50IG9yZ2FuaWMgbWF0dGVyLiBUaGUgdHVybm92ZXIgY2FuIGJlIGNvbnRyb2xsZWQgYnkgY2hhbmdpbmcgdGhlIGV4dGVybmFsIHJlc2lzdGFuY2UuIEhpZ2ggcmVzaXN0YW5jZSBsaW1pdHMgdGhlIGVsZWN0cmljYWwgcGVyZm9ybWFuY2UsIGxvdyByZXNpc3RhbmNlIGluY3JlYXNlcyBzZWRpbWVudCB0dXJub3ZlciBhbmQgcmlza3MgZGVzdGFiaWxpemluZyB0aGUgc3lzdGVtLiBUbyBhY2hpZXZlIHJlZHVjZWQgY29uY2VudHJhdGlvbnMgb2YgcGhvc3BoYXRlIGluIHRoZSB3YXRlciBjb2x1bW4sIGFub2RlcyBtdXN0IGJlIHBsYWNlZCBpbiB0aGUgc2VkaW1lbnQgc3VyZmFjZSBzaW5jZSBhIGZldyBtaWxsaW1ldHJlcyBvZiBzZWRpbWVudCBtYXkgY29udGFpbiBhbmQgcmVsZWFzZSBlbm91Z2ggUCB0byBzdXN0YWluIGV1dHJvcGhpY2F0aW9uLiBQaG9zcGhhdGUgd2FzIG1lYXN1cmVkIGluIHRoZSB3YXRlciBjb2x1bW4gbmVhciB0aGUgc2VkaW1lbnQsIGluIHN5c3RlbXMgd2l0aCBhbmQgd2l0aG91dCBlbGVjdHJvZGVzLiBQaG9zcGhhdGUgY29uY2VudHJhdGlvbnMgd2VyZSBzdWNjZXNzZnVsbHkgcmVkdWNlZCBieSB1cCB0byA5NSAlIGluIDItY2hhbWJlciBsYWJvcmF0b3J5IHJlYWN0b3JzLCBhbmQgYnkgdXAgdG8gOTggJSBpbiB0aGUgcGlsb3QgaW5zdGFsbGF0aW9uLiBJbiB0aGUgcGlsb3QgZXhwZXJpbWVudCwgcGhvc3BoYXRlIGNvbmNlbnRyYXRpb25zIGFib3ZlIGVsZWN0cm9kZXMgd2VyZSBsb3dlciBpbiBhbGwgbWVhc3VyZW1lbnRzLCBjb21wYXJlZCB0byBjb250cm9scy4gU2VkaW1lbnQgcmVkb3ggcG90ZW50aWFscyBpbmNyZWFzZWQgZnJvbSAtNDAwIG1WIHZzIHN0YW5kYXJkIGh5ZHJvZ2VuIGVsZWN0cm9kZSAoU0hFKSB0byAtMTc1IG1WIHZzIFNIRSwgY29ycmVzcG9uZGluZyB0byBhIG1vdmUgZnJvbSBhIG1ldGhhbm9nZW5ldGljIGRvbWFpbiB0byBzdWxwaHVyIHJlZHVjaW5nLiBBbmFseXNpcyBvZiB0aGUgbWljcm9iaWFsIGNvbW11bml0eSBpbmRpY2F0ZWQgZW5oYW5jZWQgc3VscGh1ciBjeWNsaW5nIHdoZXJlIGVsZWN0cm9kZXMgb3hpZGl6ZSBzdWxwaGlkZSB3aGljaCB0aGUgYmFjdGVyaWEgYWdhaW4gcmVkdWNlIHRvIG9idGFpbiBlbmVyZ3kuIEl0IHdhcyBoeXBvdGhlc2lzZWQgdGhhdCBTTUZDIHByb21vdGUgaGlnaGVyIHJlZG94IHBvdGVudGlhbHMgYnkgZmFjaWxpdGF0aW5nIG94aWRhdGlvbiBvZiBzdWxwaHVyIGFuZCBpcm9uIHNwZWNpZXMsIGluaGliaXRpbmcgSDJTIGZvcm1hdGlvbiBhbmQgRmUgc2NhdmVuZ2luZyBieSBGZVMgZm9ybWF0aW9uLCB0aHVzIGFsbG93aW5nIGZvciBQIGJpbmRpbmcuIEEgbG93ZXIgcmVsYXRpdmUgYWJ1bmRhbmNlIG9mIG1ldGhhbm9nZW5zIG9uIGVsZWN0cm9kZXMgY29tcGFyZWQgdG8gY29udHJvbHMgc3VnZ2VzdHMgc3VwcHJlc3Npb24gb3Igb3V0Y29tcGV0aW5nIG9mIG1ldGhhbm9nZW5zLlF1YW50aWZpY2F0aW9uIG9mIG1ldGhhbmUgcHJvZHVjdGlvbiB3YXMgZG9uZSBvbiB0aGUgYmFzaXMgb2YgYSBDSDQ6Q08yIHJhdGlvIHRvIGFsbG93IGNvbXBhcmFiaWxpdHkgYmV0d2VlbiBzeXN0ZW1zLiBNZXRoYW5lIGVtaXNzaW9uIHdhcyByZWR1Y2VkIGJ5IHVwIHRvIDk5ICUgYW5kIG1haW50YWluZWQgaW5oaWJpdGlvbiBmb3Igc2V2ZXJhbCBtb250aHMgaW4gbGFib3JhdG9yeSBzeXN0ZW1zIHdpdGggZWxlY3Ryb2Rlcy4gSW4gdGhlIHBpbG90IGluc3RhbGxhdGlvbnMgbm8gZ2FzIHNhbXBsZXMgY291bGQgYmUgY29sbGVjdGVkLCBhbmQgbm8gQ08yOkNINCByYXRpb3Mgd2VyZSBvYnRhaW5lZC4gQWVyYXRpb24gaXMgdGhlIGN1cnJlbnQgcmVzdG9yYXRpb24gdGVjaG5pcXVlIHRoYXQgbW9zdCBjbG9zZWx5IHJlc2VtYmxlcyB0aGUgcHJvcG9zZWQgU01GQyB0ZWNobmlxdWUuIEFlcmF0aW9uIGhhcyBwcmV2aW91c2x5IGZhaWxlZCB0byByZWR1Y2UgUCByZWxlYXNlIGluIHNldmVyYWwgc2VwYXJhdGUgaW52ZXN0aWdhdGlvbnMuIFNNRkNzIGFzIGEgcmVtZWRpYXRpb24gdG9vbCB3YXMgZm91bmQgdG8gb2ZmZXIgc2V2ZXJhbCBiZW5lZml0cyBvdmVyIGNvbnZlbnRpb25hbCBtZXRob2RzOiBTTUZDcyBvZmZlcnMgY29udGludW91cyBlZmZlY3QsIHdoaWxlIG90aGVyIHRlY2huaXF1ZXMgcmVxdWlyZSBjb250aW51b3VzIG9yIHJlcGVhdGVkIHRyZWF0bWVudC4gU01GQ3MgY2FuIGJlIGNvbnRyb2xsZWQgaW4gcmVhbC10aW1lIGJ5IGFsdGVyaW5nIHRoZSBleHRlcm5hbCByZXNpc3RvciBhbmQgY2FuIGJlIGVhc2lseSBtb25pdG9yZWQgYnkgbWVhc3VyaW5nIHRoZSBwcm9kdWNlZCBlbGVjdHJpY2FsIG91dHB1dC4gSWYgaW1wbGVtZW50ZWQgYXQgbGFyZ2Ugc2NhbGUgaXQgaXMgZXhwZWN0ZWQgdGhhdCB0aGUgZWxlY3Ryb2RlIGZpZWxkcyB3aWxsIGhlbHAgbWFpbnRhaW4gbG93IHBob3NwaGF0ZSBjb25jZW50cmF0aW9ucyBpbiB0aGUgd2F0ZXIgZm9yIGRlY2FkZXMgd2l0aCBtaW5pbWFsIGludGVydmVudGlvbiwgYW5kIGFmdGVyIHNvbWUgdGltZSwgYmVudGhpYyBwbGFudHMgYXJlIGV4cGVjdGVkIHRvIHJldHVybiwgdGh1cyBtYWludGFpbmluZyB0aGUgYmFsYW5jZSBoZW5jZSBmb3J0aC4iLCJ1cmwiOiJodHRwczovL2xvY2FsLmZvcnNrbmluZ3Nwb3J0YWwuZGsvbG9jYWwvZGtpLWNnaS93cy9jcmlzLWxpbms%2Fc3JjPWR0dSZpZD1kdHUtZDY2ZDMzNzAtMjU2Zi00ODg1LWEwOWUtMzM2YTkyNGY0YmVkJnRpPVNlZGltZW50JTIwbWljcm9iaWFsJTIwZnVlbCUyMGNlbGxzJTIwZm9yJTIwbGFrZSUyMHJlc3RvcmF0aW9uIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2JhY2tlbmQub3JiaXQuZHR1LmRrL3dzL2ZpbGVzLzI0ODA2OTU4OS9UaGVzaXNfb25saW5lX3ZlcnNpb25fS2FybF9IYXh0aGF1c2VuLnBkZiIsImlzc3VlZERhdGUiOiIyMDIxLTAxLTAxIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0wOCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiUmVzZWFyY2ggUG9ydGFsIERlbm1hcmsiLCJpbXBhY3RGYWN0b3IiOiIwLjA0In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6NzZ9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiVGVjaG5pY2FsIFVuaXZlcnNpdHkgb2YgRGVubWFyayJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MCwiY2l0ZWRCeUNvdW50IjoxfX0sInF1b3RlIjoiRXh0ZXJuYWwgaW5wdXRzIGluY3JlYXNlIFAgY29uY2VudHJhdGlvbiBpbiAgdGhlIHdhdGVyLCBjYXVzaW5nIGFsZ2FsIGJsb29tcyB0aGF0IHNoYWRvdyB0aGUgbGFrZSBib3R0b20sIGFuZCBiZW50aGljIG1hYy1yb3BoeXRlcyByZWNlZGUuIFByZWRhdG9yeSBmaXNoIGFyZSB1bmFibGUgdG8gaHVudCwgYW5kIHRoZWlyIGZvb2Qgc291cmNlIGlzICBkaXNydXB0ZWQgc3VjaCBhcyBmaXNoIGZlZWRpbmcgb24gYmVudGhpYyBpbnZlcnRlYnJhdGVzIGFuZCBib3R0b20tZHdlbGxpbmcgYW5pbWFscyBhcmUgc3VmZm9jYXRlZCAoVmFuZGVyIFphbmRlbiBhbmQgVmFkZWJvbmNvZXVyIDIwMDIpLCBzbyB0aGF0IHBsYW5rdGl2b3JvdXMgZmlzaCBzcGVjaWVzIGRvbWluYXRlLiBUaGlzIGluY3JlYXNlcyBwcmVkYXRpb24gb24gem9vIHBsYW5rdG9uICBhbmQgcmVkdWNlcyBncmF6aW5nIG9uIGFsZ2FlLiBBbGdhZSBwcm9saWZlcmF0ZSwgZGllLCBhbmQgc2luayB0byB0aGUgYm90dG9tLCAgY29uc3VtaW5nIG94eWdlbiB3aGVuIGRlZ3JhZGVkLCB3aGljaCBsZWFkcyB0byBveHlnZW4gZGVmaWNpdCBpbiB0aGUgYm90LXRvbSwga2lsbGluZyBmaXNoIChXdXJ0c2JhdWdoIGV0IGFsLiAyMDE5KS4gRmlndXJlIDI6IFZpY2lvdXMgY3ljbGVzIG9mIGV1dHJvcGhpY2F0aW9uLiBMZWZ0OiBJbnRlcmxpbmtlZCBwcm9jZXNzZXMgc3VzdGFpbmluZyBldXRyb3BoaS1jYXRpb24gaW4gYSBsYWtlLiBCbHVlOiBwcm9jZXNzZXMgaW4gd2F0ZXIsIGdyZWVuOiBwcm9jZXNzZXMgaW52b2x2aW5nIG1pY3JvYWxnYWUsIGdyZXk6IHNlZC1pbWVudCByZWxhdGVkIHByb2Nlc3NlcywgeWVsbG93OiBwcm9jZXNzZXMgaW52b2x2aW5nIGZhdW5hLiBSaWdodCwgZGFzaGVkIGxpbmU6IFRoZSBvcmRlciBpbiB3aGljaCBlbGVjdHJvbiBhY2NlcHRvcnMgYXJlIGNvbnN1bWVkIGluIHRoZSBhYnNlbmNlIG9mIE8yIHNob3duIGFzIGNvbmNlbnRyYXRpb24gdmVyc3VzICBkZXB0aCBpbiBzZWRpbWVudCwgYW5kIGF0IHdoaWNoIHBvdGVudGlhbCB0aGlzIG9jY3VycyAoQmVybmVyIDE5ODEsIGJ5IGNvdXJ0ZXN5IG9mIHRoZSBzb2NpZXR5ICBmb3Igc2VkaW1lbnRhcnkgZ2VvbG9neSkuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxNywiYmJveCI6eyJ0b3AiOjc5LjkzMzE1MTI0NTExNzE5LCJsZWZ0IjoxNC4yOTU1MDM2MTYzMzMwMDgsIndpZHRoIjo3MS45Mzg5MzQzMjYxNzE4OCwiaGVpZ2h0IjoyLjA1MzQ3NzI4NzI5MjQ4MDV9fSx7InBhZ2VOdW1iZXIiOjE3LCJiYm94Ijp7InRvcCI6ODEuOTcxMzQzOTk0MTQwNjIsImxlZnQiOjE0LjI5NTUwMzYxNjMzMzAwOCwid2lkdGgiOjcxLjMzMzAwMDE4MzEwNTQ3LCJoZWlnaHQiOjIuMDUzNDc3Mjg3MjkyNDgwNX19LHsicGFnZU51bWJlciI6MTcsImJib3giOnsidG9wIjo4NC4wMjM3OTYwODE1NDI5NywibGVmdCI6MTQuMjk1NTAzNjE2MzMzMDA4LCJ3aWR0aCI6NzEuODEyMDE5MzQ4MTQ0NTMsImhlaWdodCI6Mi4wNTM0NzcyODcyOTI0ODA1fX0seyJwYWdlTnVtYmVyIjoxNywiYmJveCI6eyJ0b3AiOjg2LjA2MTk5NjQ1OTk2MDk0LCJsZWZ0IjoxNC4yOTU1MDM2MTYzMzMwMDgsIndpZHRoIjo3MS45NTczNTkzMTM5NjQ4NCwiaGVpZ2h0IjoyLjA1MzQ3NzI4NzI5MjQ4MDV9fSx7InBhZ2VOdW1iZXIiOjE4LCJiYm94Ijp7InRvcCI6OS45MjY0ODc5MjI2Njg0NTcsImxlZnQiOjE0LjI5NTUwMzYxNjMzMzAwOCwid2lkdGgiOjcxLjk3NDA2MDA1ODU5Mzc1LCJoZWlnaHQiOjEwLjIzNDc3NzQ1MDU2MTUyM319LHsicGFnZU51bWJlciI6MTgsImJib3giOnsidG9wIjo0NC43MzQwNDMxMjEzMzc4OSwibGVmdCI6MTQuMjk1NTAzNjE2MzMzMDA4LCJ3aWR0aCI6NzEuODkzNDMyNjE3MTg3NSwiaGVpZ2h0IjoxMC40MDMyODk3OTQ5MjE4NzV9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI5MyJ9fSx7Im9yaWdpbiI6NCwid29yayI6eyJ0eXBlIjo4LCJzdWJ0eXBlIjp7Im9uZW9mS2luZCI6ImFydGljbGVUeXBlIiwiYXJ0aWNsZVR5cGUiOjF9LCJjb250cmlidXRvcnMiOlt7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJIYW5zIFcuIiwiZmFtaWx5IjoiUGFlcmwifX1dLCJpZHMiOnsiaXNzbiI6W10sImlzYm4iOltdLCJkb2kiOiIxMC4xNzYxNS95bjU1LWI0MzgiLCJvcGVuYWxleCI6Ilc0MzA3MDYzNzM5In0sInRpdGxlIjoiTWl0aWdhdGluZyB0b3hpYyBwbGFua3RvbmljIGN5YW5vYmFjdGVyaWFsIGJsb29tcyBpbiBhcXVhdGljIGVjb3N5c3RlbXMgZmFjaW5nIGluY3JlYXNpbmcgYW50aHJvcG9nZW5pYyBhbmQgY2xpbWF0aWMgcHJlc3N1cmVzIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjE3NjE1L3luNTUtYjQzOCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9jZHIubGliLnVuYy5lZHUvZG93bmxvYWRzL3doMjQ3MjkwZyIsImlzc3VlZERhdGUiOiIyMDE4LTAxLTAxIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0wNiIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiQ2Fyb2xpbmEgRGlnaXRhbCBSZXBvc2l0b3J5IChVbml2ZXJzaXR5IG9mIE5vcnRoIENhcm9saW5hIGF0IENoYXBlbCBIaWxsKSIsImltcGFjdEZhY3RvciI6IjAuMTAifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiVW5pdmVyc2l0eSBvZiBOb3J0aCBDYXJvbGluYSBhdCBDaGFwZWwgSGlsbCJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MCwiY2l0ZWRCeUNvdW50IjowfX0sInF1b3RlIjoiVGhpcyBsZWFkcyB0byBjb250aW51aW5nIGRlbWFuZCBmb3IgTiBzdXBwbGllcyB0byBzdXN0YWluIHByaW1hcnkgcHJvZHVjdGlvbiBhbmQgcHJvbW90ZSBldXRyb3BoaWNhdGlvbiBbMzgsMzldLCAoMikgYSBsZW5ndGh5IGxlZ2FjeSBvZiBQIGxvYWRpbmcgdGhhdCBpcyBzdG9yZWQgaW4gc2VkaW1lbnRzLCB3aGljaCBpcyByZWFkaWx5IHJlY3ljbGVkIGFuZCBzdXBwb3J0cyBldXRyb3BoaWNhdGlvbiwgKDMpIE4yIGZpeGF0aW9uLCB3aGlsZSBwcm92aWRpbmcg4oCcbmV34oCdIE4sIGZhaWxzIHRvIG1lZXQgZWNvc3lzdGVtLXNjYWxlIE4gcmVxdWlyZW1lbnRzIFs0MCw0MV0uIEZyb20gYSBudXRyaWVudCBtYW5hZ2VtZW50IHBlcnNwZWN0aXZlLCB0aGlzIG1lYW5zIHRoYXQgY29udGludWVkIGV4dGVybmFsIGxvYWRpbmcgb2YgTiBwbGF5cyBhIGNyaXRpY2FsIHJvbGUgaW4gYWNjZWxlcmF0aW5nIGV1dHJvcGhpY2F0aW9uIGFuZCBwcm9tb3RpbmcgYWxnYWwgYmxvb21zIFszNl0uIE9uZSB0cm91YmxpbmcgaW5kaWNhdG9yIG9mIE4tZHJpdmVuIGV1dHJvcGhpY2F0aW9uIGFuZCBDeWFub0hBQnMgaW4gZnJlc2h3YXRlciBzeXN0ZW1zIGlzIHRoZSBnZW9ncmFwaGljIGV4cGFuc2lvbiBhbmQgaW5jcmVhc2UgZnJlcXVlbmN5IG9mIHRoZSBub24tTjIgZml4aW5nIHRveGluLXByb2R1Y2luZyBnZW51cyBNaWNyb2N5c3RpcyBbNDJdLCBlc3BlY2lhbGx5IGluIGxhcmdlIGxha2VzIChlLmcuLCBMYWtlIEVyaWUsIENhbmFkYS1VU0EsIExha2UgS2FzdW1hZ3VyYSwgSmFwYW4sIExha2UgT2tlZWNob2JlZSwgRmxvcmlkYSwgVVNBLCBMYWtlIFRhaWh1LCBDaGluYSkuIFRoaXMgZ2VudXMsIGFzIHdlbGwgYXMgYW5vdGhlciBwbGFua3RvbmljIG5vbi1OMiBmaXhpbmcgQ3lhbm9IQUIgUGxhbmt0b3Rocml4LCBhbmQgc2V2ZXJhbCBiZW50aGljIGFuYWxvZ3VlIGJsb29tIHRheGEgKGUuZy4sIG5vbi1OMiBmaXhpbmcgTHluZ2J5YSBhbmQgT3NjaWxsYXRvcmlhIHNwcC4pLCBzZXJ2ZSBhcyBpbmRpY2F0b3JzIG9mIE4tb3Zlci1lbnJpY2htZW50IFszNiw0Ml0uIFRoZSBwcm9saWZlcmF0aW9uIG9mIHRoZXNlIG5vbi1OMiBmaXhpbmcgQ3lhbm9IQUJzIGlzIGFuIGFkZGl0aW9uYWwgaW5kaWNhdG9yIHRoZSBOIGlucHV0IHJlZHVjdGlvbnMgYXJlIG5lZWRlZCB0byByZXZlcnNlIHRoaXMgdHJlbmQuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NzcuNDQ2MjUwOTE1NTI3MzQsImxlZnQiOjE2LjQyODUxNDQ4MDU5MDgyLCJ3aWR0aCI6NzAuNzE0NzIxNjc5Njg3NSwiaGVpZ2h0IjoxLjQ1MDc5NjQ4NDk0NzIwNDZ9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo3OS4wNTA5Nzk2MTQyNTc4MSwibGVmdCI6MTIuODU3MDYxMzg2MTA4Mzk4LCJ3aWR0aCI6NzQuNDk0OTg3NDg3NzkyOTcsImhlaWdodCI6MS40NTA3OTY0ODQ5NDcyMDQ2fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6ODAuNjU1NTg2MjQyNjc1NzgsImxlZnQiOjEyLjgwMTk2MTg5ODgwMzcxMSwid2lkdGgiOjc0LjM0MDAxOTIyNjA3NDIyLCJoZWlnaHQiOjEuNDUwNzk2NDg0OTQ3MjA0Nn19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjgyLjI2MDE4NTI0MTY5OTIyLCJsZWZ0IjoxMi44NTcwNjEzODYxMDgzOTgsIndpZHRoIjo3NC4yODUxODY3Njc1NzgxMiwiaGVpZ2h0IjoxLjY2Njk3NzY0Mzk2NjY3NDh9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo4My44NjQ5MDYzMTEwMzUxNiwibGVmdCI6MTIuODU3MDYxMzg2MTA4Mzk4LCJ3aWR0aCI6NzQuMjg1MDE4OTIwODk4NDQsImhlaWdodCI6MS40NTA3OTY0ODQ5NDcyMDQ2fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6ODUuNDY5NTEyOTM5NDUzMTIsImxlZnQiOjEyLjg1NzA2MTM4NjEwODM5OCwid2lkdGgiOjcxLjEwNDk4ODA5ODE0NDUzLCJoZWlnaHQiOjEuNDUwNzk2NDg0OTQ3MjA0Nn19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjg3LjA3NDIzNDAwODc4OTA2LCJsZWZ0IjoxNi40Mjg1MTQ0ODA1OTA4Miwid2lkdGgiOjcwLjcxNDE3MjM2MzI4MTI1LCJoZWlnaHQiOjEuNDUwNzk2NDg0OTQ3MjA0Nn19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjg4LjY3ODg0MDYzNzIwNzAzLCJsZWZ0IjoxMi44NTcwNjEzODYxMDgzOTgsIndpZHRoIjo3NC4yODYwMjYwMDA5NzY1NiwiaGVpZ2h0IjoxLjY2Njk3NzY0Mzk2NjY3NDh9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo5MC4yODM0Mzk2MzYyMzA0NywibGVmdCI6MTIuODU3MDYxMzg2MTA4Mzk4LCJ3aWR0aCI6NzQuMjg1NDE1NjQ5NDE0MDYsImhlaWdodCI6MS40NzY4MzA0ODI0ODI5MTAyfX0seyJwYWdlTnVtYmVyIjo2LCJiYm94Ijp7InRvcCI6MTAuNTA2NTEzNTk1NTgxMDU1LCJsZWZ0IjoxMi44NTcwNjEzODYxMDgzOTgsIndpZHRoIjo3NC4yMDIwMTg3Mzc3OTI5NywiaGVpZ2h0IjoxLjU4NjYwODQwOTg4MTU5MTh9fSx7InBhZ2VOdW1iZXIiOjYsImJib3giOnsidG9wIjoxMi4xMTEyMzc1MjU5Mzk5NDEsImxlZnQiOjEyLjg1NzA2MTM4NjEwODM5OCwid2lkdGgiOjc0LjI4NTY3NTA0ODgyODEyLCJoZWlnaHQiOjEuNjY2OTgxMzM5NDU0NjUwOX19LHsicGFnZU51bWJlciI6NiwiYmJveCI6eyJ0b3AiOjEzLjcxNTgzNzQ3ODYzNzY5NSwibGVmdCI6MTIuODU3MDYxMzg2MTA4Mzk4LCJ3aWR0aCI6NzQuMjg1OTcyNTk1MjE0ODQsImhlaWdodCI6MS40NzY4MzQxNzc5NzA4ODYyfX0seyJwYWdlTnVtYmVyIjo2LCJiYm94Ijp7InRvcCI6MTUuMzIwNTYxNDA4OTk2NTgyLCJsZWZ0IjoxMi44NTcwNjEzODYxMDgzOTgsIndpZHRoIjo3NC4yODUxMDI4NDQyMzgyOCwiaGVpZ2h0IjoxLjY2Njg2NTM0ODgxNTkxOH19LHsicGFnZU51bWJlciI6NiwiYmJveCI6eyJ0b3AiOjE2LjkyNTE2MzI2OTA0Mjk3LCJsZWZ0IjoxMi44NTcwNjEzODYxMDgzOTgsIndpZHRoIjo3NC4yODUzNTQ2MTQyNTc4MSwiaGVpZ2h0IjoxLjY2Njk4MTMzOTQ1NDY1MDl9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI2In19XQ%3D%3D. In peri-urban lakes like Bellandur or Hussainsagar, ag-dominated catchments contribute >60% of pollutant loads, manifesting as perennial froth and scum (Nair & Nayak, 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hdnlhIFZpa3JhbWFuIiwiZmFtaWx5IjoiTmFpciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQcmF0ZWVwIEt1bWFyIiwiZmFtaWx5IjoiTmF5YWsifX1dLCJpZHMiOnsiaXNzbiI6WyIyMjk2LTc3NDUiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzODkvZm1hcnMuMjAyMy4xMDg3Mjk2Iiwib3BlbmFsZXgiOiJXNDM4MjcyMzEzMyJ9LCJ0aXRsZSI6IlVuY292ZXJpbmcgd2F0ZXIgcXVhbGl0eSBhbmQgZXZhbHVhdGluZyB2dWxuZXJhYmlsaXRpZXMgb2Ygc21hbGwtc2NhbGUgZmlzaGVyaWVzIGluIENoaWxpa2EgTGFnb29uLCBJbmRpYSIsImFic3RyYWN0IjoiU21hbGwtc2NhbGUgZmlzaGVyaWVzIChTU0ZzKSBwbGF5IGEgc3Ryb25nIHJvbGUgaW4gc3VzdGFpbmluZyBtaWxsaW9ucyBvZiBsaXZlbGlob29kcywgZm9vZCBzZWN1cml0eSwgbnV0cml0aW9uLCBhbmQgaW5jb21lIGdsb2JhbGx5IGJ1dCB0aGUgZmlzaGVycyBlbmdhZ2VkIGluIHRoaXMgc2VjdG9yIHNpbXVsdGFuZW91c2x5IGV4cGVyaWVuY2UgaGlnaCBsZXZlbHMgb2YgdnVsbmVyYWJpbGl0eSBhbmQgcHJvY2Vzc2VzIG9mIG1hcmdpbmFsaXNhdGlvbi4gU2V2ZXJhbCBmYWN0b3JzIGFyZSBhdHRyaWJ1dGVkIHRvIHRoZSBtdWx0aWRpbWVuc2lvbmFsIHZ1bG5lcmFiaWxpdGllcyB0aGUgc21hbGwtc2NhbGUgZmlzaGVycyBleHBlcmllbmNlLCBzcGFubmluZyBib3RoIG5hdHVyYWwgKGUuZy4sIG5hdHVyYWwgZGlzYXN0ZXJzLCBlY29zeXN0ZW0gY2hhbmdlKSBhbmQgYW50aHJvcG9nZW5pYyAoZS5nLiwgcG9saWN5IGNoYW5nZSwgaHlkcm9sb2dpY2FsIGludGVydmVudGlvbnMsIGFxdWFjdWx0dXJlKSBwcmVzc3VyZXMuIFdoaWxlIHRoZXJlIGlzIG11Y2ggbGl0ZXJhdHVyZSBvbiB2YXJpb3VzIG5hdHVyYWwgYW5kIGh1bWFuIGRyaXZlcnMgb2YgdnVsbmVyYWJpbGl0eSBpbiBzbWFsbC1zY2FsZSBmaXNoZXJ5IGNvbW11bml0aWVzLCBhbiBhYnNlbmNlIG9mIHJlc2VhcmNoIGNvbm5lY3RpbmcgdnVsbmVyYWJpbGl0eSB3aXRoIHdhdGVyIHF1YWxpdHkgaXMgZXZpZGVudC4gRmlzaGVyIGNvbW11bml0aWVzIG9mdGVuIHRhbGsgYWJvdXQgZmlzaCBpbiByZWxhdGlvbiB0byB0aGUgaGVhbHRoIG9mIHRoZWlyIGFxdWF0aWMgaGFiaXRhdHMgd2hlcmVpbiB3YXRlciBxdWFsaXR5IGlzIHNlZW4gYXMgYSBrZXkgcGFyYW1ldGVyLiBUaGUgbGluayBiZXR3ZWVuIGhlYWx0aHkgZmlzaCBhbmQgZ29vZCBxdWFsaXR5IHdhdGVyIGhhcyBzaWduaWZpY2FudCBpbXBsaWNhdGlvbnMgZm9yIHN0cm9uZyBhbmQgdmlhYmxlIGZpc2hpbmcgY29tbXVuaXRpZXMuIFRoaXMgcGFwZXIgZXhhbWluZXMgdGhlc2UgbGlua3MgZnVydGhlciBieSBmb2N1c2luZyBvbiB0aGUgbmF0dXJlIG9mIHZ1bG5lcmFiaWxpdGllcyBjYXVzZWQgYnkgd2F0ZXIgcXVhbGl0eSBjaGFuZ2VzIGluIHRoZSBzbWFsbC1zY2FsZSBmaXNoZXJ5IHN5c3RlbSBvZiBDaGlsaWthIExhZ29vbiBpbiBJbmRpYS4gV2UgdW5kZXJ0YWtlIGRldGFpbGVkIGFuYWx5c2lzIG9mIHRoZSBpbnZhc2l2ZSBzaHJpbXAgYXF1YWN1bHR1cmUgYWN0aXZpdGllcyBhbmQgaHlkcm9sb2dpY2FsIGludGVydmVudGlvbnMgZm9yIG9wZW5pbmcgb2YgYSBsYWdvb24gaW5sZXQgd2l0aCB0aGUgQmF5IG9mIEJlbmdhbCBhcyB0d28gZG9taW5hbnQgZHJpdmVycyBhZHZlcnNlbHkgaW1wYWN0aW5nIHdhdGVyIHF1YWxpdHkgYW5kIGluY3JlYXNpbmcgdnVsbmVyYWJpbGl0aWVzIG9mIHRoZSBlbnRpcmUgc21hbGwtc2NhbGUgZmlzaGVyaWVzIHNvY2lhbC1lY29sb2dpY2FsIHN5c3RlbS4gT3VyIGFuYWx5c2lzIHN1Z2dlc3RzIHRoYXQgdGhlcmUgYXJlIHN0cm9uZyBpbnRlcmNvbm5lY3Rpb25zIGJldHdlZW4gY2hhbmdlcyBpbiB3YXRlciBxdWFsaXR5IGFuZCB0aGUgbGV2ZWxzIG9mIHZ1bG5lcmFiaWxpdGllcyBpbiB0aGUgU1NGcyBvZiBDaGlsaWthIExhZ29vbi4gUG9sbHV0YW50cyBzdWNoIGFzIHBlc3RpY2lkZXMsIGFuZCBvcmdhbmljIGNvbXBvdW5kcyBhY2N1bXVsYXRlIGluIGZpc2ggdGlzc3VlcyBhbmQgYWZmZWN0IHRoZWlyIGdyb3d0aCwgcmVwcm9kdWN0aW9uLCBhbmQgb3ZlcmFsbCBoZWFsdGguIFRoaXMgbGVkIHRvIGRlY2xpbmVzIGluIGZpc2ggcG9wdWxhdGlvbnMsIG1ha2luZyBpdCBtb3JlIGRpZmZpY3VsdCBmb3IgZmlzaGVycyB0byBtYWtlIGEgbGl2aW5nLiBJbiBhZGRpdGlvbiB0byBkaXJlY3QgaW1wYWN0cyBvbiBmaXNoIHBvcHVsYXRpb25zLCBwb29yIHdhdGVyIHF1YWxpdHkgYWxzbyBoYXMgaW5kaXJlY3QgZWZmZWN0cyBvbiB0aGUgc29jaWFsIGFuZCBlY29ub21pYyB2dWxuZXJhYmlsaXR5IG9mIFNTRnMuIEZvciBleGFtcGxlLCBjb250YW1pbmF0aW9uIG9mIHdhdGVyIHNvdXJjZXMgbGVkIHRvIHRoZSByZWR1Y2VkIG51bWJlciBvZiBmaXNoIHNwZWNpZXMgcmVkdWNpbmcgdGhlIGFtb3VudCBvZiB0aW1lIGZpc2hlcnMgY2FuIHNwZW5kIG9uIGZpc2hpbmcgYWN0aXZpdGllcy4gVGhpcyBhbHNvIGFmZmVjdGVkIHRoZSBtYXJrZXRhYmlsaXR5IG9mIGZpc2ggcHJvZHVjdHMsIHJlZHVjZWQgaW5jb21lIGFuZCBpbmNyZWFzZWQgcG92ZXJ0eS4gVG8gZnVsbHkgdW5kZXJzdGFuZCB0aGUgaW50ZXJjb25uZWN0aW9ucyBiZXR3ZWVuIHdhdGVyIHF1YWxpdHkgYW5kIHZ1bG5lcmFiaWxpdHkgaW4gU1NGcyBpbiBDaGlsaWthIExhZ29vbiwgaXQgaXMgaW1wb3J0YW50IHRvIGNvbnNpZGVyIGJvdGggZW52aXJvbm1lbnRhbCBhbmQgc29jaWFsIGZhY3RvcnMsIGFzIHdlbGwgYXMgdGhlIGNvbXBsZXggZmVlZGJhY2sgbG9vcHMgYmV0d2VlbiB0aGVzZSBmYWN0b3JzLiBUaGUgc3R1ZHkgaGVscHMgaW4gYnJpZGdpbmcgYSBjcnVjaWFsIGdhcCBpbiBvdXIgdW5kZXJzdGFuZGluZyBvZiB0aGUgcm9sZSBvZiB3YXRlciBxdWFsaXR5IGluIHZ1bG5lcmFiaWxpdHkgYW5hbHlzaXMgd2l0aGluIHJlc291cmNlIGRlcGVuZGVudCBjb21tdW5pdGllcy4gV2UgY29uY2x1ZGUgd2l0aCBrZXkgaW5zaWdodHMgb24gcG9zc2libGUgY29waW5nIHJlc3BvbnNlcyBhbmQgYWRhcHRpdmUgY2FwYWNpdHkgbmVjZXNzYXJ5IGZvciB0aGUgc21hbGwtc2NhbGUgZmlzaGVyaWVzIGNvbW11bml0aWVzIHRvIHRyYW5zaXRpb24gdG93YXJkIHZpYWJpbGl0eS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMzM4OS9mbWFycy4yMDIzLjEwODcyOTYiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vd3d3LmZyb250aWVyc2luLm9yZy9hcnRpY2xlcy8xMC4zMzg5L2ZtYXJzLjIwMjMuMTA4NzI5Ni9wZGYiLCJpc3N1ZWREYXRlIjoiMjAyMy0wNi0yOSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZyb250aWVycyBpbiBNYXJpbmUgU2NpZW5jZSIsInZvbHVtZSI6MTAsImltcGFjdEZhY3RvciI6IjQuMjgifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRnJvbnRpZXJzIE1lZGlhIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo1MSwiY2l0ZWRCeUNvdW50Ijo1fX0sInF1b3RlIjoiV2F0ZXIgcXVhbGl0eSBvZiBDaGlsaWthIExhZ29vbiBoYXMgZGVncmFkZWQgZHVlIHRvIGFudGhyb3BvZ2VuaWMgaW50ZXJ2ZW50aW9ucyBhbmQgbmF0dXJhbCBmb3JjZXMgb2YgY2hhbmdlLiBWYXJpb3VzIGFjdGl2aXRpZXMgaW4gdGhlIENoaWxpa2EgTGFnb29uLCBzdWNoIGFzIGluZHVzdHJpYWwgd2FzdGV3YXRlciBkaXNwb3NhbCwgc2V3YWdlIGR1bXBpbmcsIGFxdWFjdWx0dXJlLCBoeWRyb2xvZ2ljYWwgaW50ZXJ2ZW50aW9ucywgYW5kIGN5Y2xvbmVzLCBhcmUgcHJvZHVjaW5nIHNhbGluaXR5IGNoYW5nZXMsIHNlZGltZW50IGRlcG9zaXRpb24sIG51dHJpZW50IGVucmljaG1lbnQsIGV1dHJvcGhpY2F0aW9uLCBhbmQgZGVhZCB6b25lcywgYXMgc3RhdGVkLiBGaXNoIHN1cnZpdmFsIGlzIGF0IHJpc2sgYXMgYSByZXN1bHQgb2YgdGhlIHNpZ25pZmljYW50IHJlZHVjdGlvbiBpbiBlbnZpcm9ubWVudGFsIGNpcmN1bXN0YW5jZXMsIHdoaWNoIGRpbWluaXNoZXMgZmlzaGVybWVu4oCZcyBpbmNvbWUgYW5kIHB1c2hlcyB0aGVtIG91dCBvZiBzb2NpZXR5LiBUaGVzZSBjYXVzZXMgYWZmZWN0aW5nIHRoZSB3YXRlciBxdWFsaXR5IG9mIHRoZSBMYWdvb24gZGVwaWN0IG1hbnkgZmFjZXMgb2YgdnVsbmVyYWJpbGl0eSBpbiBTU0YgY29tbXVuaXRpZXMuIEFzIHRoZSBDaGlsaWthIExhZ29vbiBlbnZpcm9ubWVudCBpcyBleHBlY3RlZCB0byB1bmRlcmdvIGluY3JlYXNpbmdseSBkcmFzdGljIGNoYW5nZXMgKHN1Y2ggYXMgY2F0Y2ggY29tcG9zaXRpb24sIGNhdGNoIGNhcGFjaXR5LCBiaW9kaXZlcnNpdHkgdmFyaWF0aW9ucywgd2F0ZXIgcXVhbGl0eSwgYW5kIGZpc2hlcnkgcmV2ZW51ZXMpLCBpbnRlcm5hdGlvbmFsIHBvbGljeSBoYXMgYmVjb21lIG1vcmUgaW50ZXJlc3RlZCBpbiBhZGFwdGF0aW9uIG1lYXN1cmVzLiBHaXZlbiBob3cgZGl2ZXJzZSBTU0YgZWNvc3lzdGVtcyBhcmUgZXhwZWN0ZWQgdG8gYmUgYmV0d2VlbiBjb250aW5lbnRzLCB0aGVyZSBpcyBubyDigJxvbmUgc2l6ZSBmaXRzIGFsbOKAnSBhcHByb2FjaC4gVGhlIHBhcGVyIG91dGxpbmVkIHNldmVyYWwgY29waW5nIGFuZCBhZGFwdGF0aW9uIG1lY2hhbmlzbXMgdG8gaW1wcm92ZSB3YXRlciBxdWFsaXR5IGFuZCBjb250aW51ZSBmaXNoaW5nIGluIGEgc3VzdGFpbmFibGUgd2F5LiBBZGFwdGF0aW9uIHN0cmF0ZWdpZXMgY2FuIG9jY2FzaW9uYWxseSBoYXZlIHVuZm9yZXNlZW4gY29uc2VxdWVuY2VzIGZvciBmaXNoZXJpZXMgYW5kIGZpc2hpbmcgY29tbXVuaXRpZXMuIEFueSB1bmV4cGVjdGVkIGNvbnNlcXVlbmNlcyBvZiBhZGFwdGF0aW9uIGNhbiBiZSBtaXRpZ2F0ZWQgdGhyb3VnaCBsb25nLXRlcm0gcGxhbm5pbmcgYW5kIGlkZW50aWZpY2F0aW9uIG9mIGZ1dHVyZSBvcHRpb25zIHRocm91Z2ggc2NlbmFyaW8gYW5hbHlzaXMuIFRoZSBmaW5kaW5ncyBvZiB0aGlzIHBhcGVyIGNhbiBiZSB1dGlsaXplZCB0byBkZXZlbG9wIHBvbGljaWVzIHRvIHByb3RlY3Qgd2F0ZXIgcXVhbGl0eSBhbmQgU1NGcyBpbiBhIHZhcmlldHkgb2YgY29hc3RhbCBsYWdvb24gc2V0dGluZ3MuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjIyLjI4NTAzNjA4NzAzNjEzMywibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ5MzA1MzQzNjI3OTMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjIzLjc3MzUxOTUxNTk5MTIxLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDgzMzY0MTA1MjI0NjEsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjI1LjI1NTU4NDcxNjc5Njg3NSwibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ4OTU1OTE3MzU4Mzk4NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjYuNzQ0MDY4MTQ1NzUxOTUzLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNTA2ODA1NDE5OTIxODc1LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyOC4yMjYxMzMzNDY1NTc2MTcsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40OTA0NjMyNTY4MzU5NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjkuNzE0NjE2Nzc1NTEyNjk1LCJsZWZ0Ijo4LjMzMTU5MjU1OTgxNDQ1Mywid2lkdGgiOjQwLjQ5NDA5ODY2MzMzMDA4LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozMS4xOTY2ODM4ODM2NjY5OTIsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40OTQzOTIzOTUwMTk1MywiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MzIuNjg1MTY1NDA1MjczNDQsImxlZnQiOjguMzMxNTk2Mzc0NTExNzE5LCJ3aWR0aCI6NDAuNDg3NDQ1ODMxMjk4ODMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM0LjE2NzIzMjUxMzQyNzczNCwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40OTc3NDU1MTM5MTYwMTYsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM1LjY1NTcxNTk0MjM4MjgxLCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ5ODYzODE1MzA3NjE3LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozNy4xMzc3ODMwNTA1MzcxMSwibGVmdCI6MTAuNjg1MTg2Mzg2MTA4Mzk4LCJ3aWR0aCI6MzguMTQxMTA1NjUxODU1NDcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM4LjYyNjI2NjQ3OTQ5MjE5LCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4NzE1MjA5OTYwOTM3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDAuMTA4MzMzNTg3NjQ2NDg0LCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4OTg1NjcxOTk3MDcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQxLjU5NjgxMzIwMTkwNDMsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDg4NjY2NTM0NDIzODMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQzLjA3ODg4MDMxMDA1ODU5NCwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40ODE4NjExMTQ1MDE5NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDQuNTY3MzYzNzM5MDEzNjcsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDgzMDU4OTI5NDQzMzYsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQ2LjA0OTQzMDg0NzE2Nzk3LCJsZWZ0Ijo4LjMzMzEwODkwMTk3NzUzOSwid2lkdGgiOjQwLjUwMDc0NzY4MDY2NDA2LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo0Ny41Mzc5MTQyNzYxMjMwNSwibGVmdCI6OC4zMzMxMDg5MDE5Nzc1MzksIndpZHRoIjo0MC40OTU2MDU0Njg3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDkuMDE5OTgxMzg0Mjc3MzQ0LCJsZWZ0Ijo4LjMzMzEwNjA0MDk1NDU5LCJ3aWR0aCI6NDAuNTA2ODE2ODY0MDEzNjcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUwLjUwODQ2NDgxMzIzMjQyLCJsZWZ0Ijo4LjMzMzEwNjA0MDk1NDU5LCJ3aWR0aCI6NDAuNDg4NTA2MzE3MTM4NjcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUxLjk5MDUyODEwNjY4OTQ1LCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDk3NzI2NDQwNDI5NjksImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUzLjQ3OTAxMTUzNTY0NDUzLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6MzMuMTU5NDQ2NzE2MzA4NTk0LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo1NC45NjEwNzg2NDM3OTg4MywibGVmdCI6MTAuNjgzNjcwMDQzOTQ1MzEyLCJ3aWR0aCI6MzguMTM2NTk2Njc5Njg3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NTYuNDQ5NTYyMDcyNzUzOTA2LCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDg3MTQ0NDcwMjE0ODQ0LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEzIn19XQ%3D%3D.

This runoff cascade amplifies eutrophication's feedbacks: blooms block light (<1 m Secchi depth), crash zooplankton by 50-80%, and upon decay, deplete DO to <2 mg/L, fostering dead zones (Jilbert et al., 2020; Sitote & Gebremedhine, 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRvbSIsImZhbWlseSI6IkppbGJlcnQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUmFvdWzigJBNYXJpZSIsImZhbWlseSI6IkNvdXR1cmUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQnJpYW4iLCJmYW1pbHkiOiJIdXNlciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLYWxldmkiLCJmYW1pbHkiOiJTYWxvbmVuIn19XSwiaWRzIjp7Imlzc24iOlsiMDAxOC04MTU4IiwiMTU3My01MTE3Il0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDA3L3MxMDc1MC0wMjAtMDQ0NTcteCIsIm1hZyI6IjMxMDA0NTkwNTAiLCJvcGVuYWxleCI6IlczMTAwNDU5MDUwIn0sInRpdGxlIjoiUHJlZmFjZTogUmVzdG9yYXRpb24gb2YgZXV0cm9waGljIGxha2VzOiBjdXJyZW50IHByYWN0aWNlcyBhbmQgZnV0dXJlIGNoYWxsZW5nZXMiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAwNy9zMTA3NTAtMDIwLTA0NDU3LXgiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vbGluay5zcHJpbmdlci5jb20vY29udGVudC9wZGYvMTAuMTAwNy9zMTA3NTAtMDIwLTA0NDU3LXgucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjAtMTEtMTYiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA3IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJIeWRyb2Jpb2xvZ2lhIiwiaXNzdWUiOjIxLCJ2b2x1bWUiOjg0NywiaW1wYWN0RmFjdG9yIjoiMy4wMCJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjQzNDMsImxhc3RQYWdlIjo0MzU3fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlNwcmluZ2VyIFNjaWVuY2UrQnVzaW5lc3MgTWVkaWEifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjk1LCJjaXRlZEJ5Q291bnQiOjU4fX0sInF1b3RlIjoiU2ltaWxhciBkZWdyYWRhdGlvbiBvZiBmcmVzaHdhdGVyIGVjb3N5c3RlbXMgaXMgb2JzZXJ2ZWQgd29ybGR3aWRlLCB3aXRoIHRoZSB0aW1lbGluZSBvZiBodW1hbiBwcmVzc3VyZSBkaWN0YXRlZCBieSB0cmVuZHMgaW4gdXJiYW5pemF0aW9uIGFuZCBpbnRlbnNpdmUgYWdyaWN1bHR1cmUgaW4gZGlmZmVyZW50IGdlb2dyYXBoaWMgcmVnaW9ucyAoZS5nLiwgVmVubmVtbyBldCBhbC4sIDIwMDkpLiBUaGUgcHJpbmNpcGFsIGRyaXZlciBvZiBlY29zeXN0ZW0gc3RyZXNzIGluIG1vc3QgYXF1YXRpYyBzeXN0ZW1zIGlzIGV1dHJvcGhpY2F0aW9uLCB0aGUgZXhjZXNzIGdyb3d0aCBvZiBhbGdhZSBkcml2ZW4gYnkgYW50aHJvcG9nZW5pYyBpbnB1dHMgb2YgdGhlIG1hY3JvbnV0cmllbnRzIG5pdHJvZ2VuIChOKSBhbmQgcGhvc3Bob3J1cyAoUCkgKFNtaXRoLCAyMDAzOyBCaXJrIGV0IGFsLiwgMjAyMCkuIE51dHJpZW50cyBhcmUgZW1pdHRlZCBmcm9tIGJvdGggcG9pbnQgYW5kIGRpZmZ1c2Ugc291cmNlcyBpbiB0aGUgaHVtYW4gZW52aXJvbm1lbnQgYW5kIGVudGVyIGFxdWF0aWMgc3lzdGVtcyB2aWEgc3VyZmFjZSBydW5vZmYsIGFzIHdlbGwgYXMgZ3JvdW5kLXdhdGVyIGFuZCBhdG1vc3BoZXJpYyBkZXBvc2l0aW9uIChQYWVybCwgMTk5NykuIEV1dHJvcGhpY2F0aW9uIG9mdGVuIGxlYWRzIHRvIHRoZSBkb21pbmFuY2Ugb2YgcHJpbWFyeSBwcm9kdWN0aW9uIGJ5IGN5YW5vYmFjdGVyaWEsIGluY2x1ZGluZyB0b3hpYyBnZW5lcmEgc3VjaCBhcyBEb2xpY2hvc3Blcm11bSwgQXBoYW5pem9tZW5vbiwgYW5kIE1pY3JvY3lzdGlzIChIdWlzbWFuIGV0IGFsLiwgMjAxOCksIGFzIHdlbGwgYXMgb3h5Z2VuIGRlcGxldGlvbiBhbmQgaGFiaXRhdCBkZWdyYWRhdGlvbiBpbiBsYWtlcyAoQXJlbmQgZXQgYWwuLCAyMDExKS4gQmVjYXVzZSBsYWtlcyBwcm92aWRlIGEgcmFuZ2Ugb2YgY3J1Y2lhbCBlY29zeXN0ZW0gc2VydmljZXMgKFdpbHNvbiAmIENhcnBlbnRlciwgMTk5OSksIHRoZXJlIGlzIGEgZ3Jvd2luZyBuZWVkIGZvciBtYW5hZ2VtZW50IGFwcHJvYWNoZXMgdGhhdCByZXN0b3JlIHdhdGVyIHF1YWxpdHkgaW4gZnJlc2h3YXRlciBzeXN0ZW1zLiBXaXRoaW4gdGhlIGZyYW1ld29yayBvZiBVbml0ZWQgTmF0aW9ucyBTdXN0YWluYWJsZSBEZXZlbG9wLW1lbnQgR29hbCA2IChDbGVhbiBXYXRlciBhbmQgU2FuaXRhdGlvbiksIHNldmVyYWwgZGlzdGluY3QgdGFyZ2V0cyByZWxhdGUgdG8gdGhlIGltcHJvdmVtZW50IG9mIHdhdGVyIHF1YWxpdHkgYW5kIHByb3RlY3Rpb24gb2YgYXF1YXRpYyBlY29zeXN0ZW1zIGJ5IDIwMzAgKFVuaXRlZCBOYXRpb25zLCAyMDE1KS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyOC4yOTg4NjQzNjQ2MjQwMjMsImxlZnQiOjguNTQ5MjA1NzgwMDI5Mjk3LCJ3aWR0aCI6MzkuMzcxNDk0MjkzMjEyODksImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MjkuOTkxMTM0NjQzNTU0Njg4LCJsZWZ0Ijo4LjU0OTIwNTc4MDAyOTI5Nywid2lkdGgiOjM5LjM2NTExOTkzNDA4MjAzLCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMxLjY4MzQwNjgyOTgzMzk4NCwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNjk2NjMyMzg1MjUzOSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozMy4zNjc5NzMzMjc2MzY3MiwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNzU2NzkwMTYxMTMyOCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNS4wNjAyNDkzMjg2MTMyOCwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNzA5NjQwNTAyOTI5NywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNi43NTI1MTc3MDAxOTUzMSwibGVmdCI6OC41NDkyMDg2NDEwNTIyNDYsIndpZHRoIjozOS4zNzY5NjA3NTQzOTQ1MywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC40MzcwODgwMTI2OTUzMSwibGVmdCI6OC41NDkyMDg2NDEwNTIyNDYsIndpZHRoIjozOS4zNjY1Njk1MTkwNDI5NywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0MC4xMjkzNTYzODQyNzczNDQsImxlZnQiOjguNTQ5MjA4NjQxMDUyMjQ2LCJ3aWR0aCI6MzkuMzY0Mzc5ODgyODEyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0MS44MjE2MjQ3NTU4NTkzNzUsImxlZnQiOjguNTQ5MjA4NjQxMDUyMjQ2LCJ3aWR0aCI6MzkuMzY1NjczMDY1MTg1NTUsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDMuNTA2MTk1MDY4MzU5Mzc1LCJsZWZ0Ijo4LjU0OTM4OTgzOTE3MjM2Mywid2lkdGgiOjM5LjM3MDM5MTg0NTcwMzEyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0NS4xOTg0NjM0Mzk5NDE0MDYsImxlZnQiOjguNTQ5NTcyOTQ0NjQxMTEzLCJ3aWR0aCI6MzkuMzY1ODQ0NzI2NTYyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ni44OTA3MzU2MjYyMjA3LCJsZWZ0Ijo4LjU0OTU3Mjk0NDY0MTExMywid2lkdGgiOjM5LjM3Mjk1NTMyMjI2NTYyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0OC41NzUzMDIxMjQwMjM0NCwibGVmdCI6OC41NDk1NzI5NDQ2NDExMTMsIndpZHRoIjozOS4zNzI0MjEyNjQ2NDg0NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1MC4yNjc1NzgxMjUsImxlZnQiOjguNTQ5NTc2NzU5MzM4Mzc5LCJ3aWR0aCI6MzkuMzcxMTI0MjY3NTc4MTI1LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjUxLjk1OTg0MjY4MTg4NDc2NiwibGVmdCI6OC41NDk1NzY3NTkzMzgzNzksIndpZHRoIjozOS4zNjc4MzU5OTg1MzUxNTYsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTMuNTc0MTE5NTY3ODcxMDk0LCJsZWZ0Ijo4LjU0OTU3Njc1OTMzODM3OSwid2lkdGgiOjM5LjM3MDc3NzEzMDEyNjk1LCJoZWlnaHQiOjEuMzM2ODk1NTg1MDYwMTE5Nn19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjU1LjI2NjM5MTc1NDE1MDM5LCJsZWZ0Ijo4LjU0OTc2MDgxODQ4MTQ0NSwid2lkdGgiOjM5LjM2NjU4MDk2MzEzNDc2NiwiaGVpZ2h0IjoxLjMzNjg5NTU4NTA2MDExOTZ9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1Ny4wMjg5NTM1NTIyNDYwOTQsImxlZnQiOjguNTQ5OTQ0ODc3NjI0NTEyLCJ3aWR0aCI6MzkuMzYyMDI2MjE0NTk5NjEsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTguNzEzNTIzODY0NzQ2MDk0LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjkuMjk5OTgyMDcwOTIyODUyLCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjYwLjQwNTc5MjIzNjMyODEyNSwibGVmdCI6MTAuNjIyNDU1NTk2OTIzODI4LCJ3aWR0aCI6MzcuMjg3ODY4NDk5NzU1ODYsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NjIuMDk4MDYwNjA3OTEwMTU2LCJsZWZ0Ijo4LjU0OTk0MzkyMzk1MDE5NSwid2lkdGgiOjM5LjM3MzMyNTM0NzkwMDM5LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjYzLjc4MjYzMDkyMDQxMDE1NiwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzMxMjY5ODM2NDI1OCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2NS40NzQ5MDY5MjEzODY3MiwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNjUxMTk5MzQwODIwMywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2Ny4xNjcxNzUyOTI5Njg3NSwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNjk0ODAxMzMwNTY2NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2OC43ODE0NDgzNjQyNTc4MSwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzczMzg0MDk0MjM4MywiaGVpZ2h0IjoxLjMzNjg5MTUzMTk0NDI3NX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjcwLjU0NDAxMzk3NzA1MDc4LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjM5LjM2OTMwODQ3MTY3OTY5LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjcyLjIzNjI4OTk3ODAyNzM0LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjM5LjM3MjAzMjE2NTUyNzM0NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo3My45MjA4NDUwMzE3MzgyOCwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzEzMDczNzMwNDY4NzUsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNDM0NCJ9fSx7Im9yaWdpbiI6NSwid29yayI6eyJ0eXBlIjo4LCJzdWJ0eXBlIjp7Im9uZW9mS2luZCI6ImFydGljbGVUeXBlIiwiYXJ0aWNsZVR5cGUiOjF9LCJjb250cmlidXRvcnMiOlt7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJZYWJzcmEgTWVsYWsiLCJmYW1pbHkiOiJTaXRvdGUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTWljaGFlbCBHaXJpbWF5IiwiZmFtaWx5IjoiR2VicmVtZWRoaW5lIn19XSwiaWRzIjp7Imlzc24iOltdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMjA5NDQvcHJlcHJpbnRzMjAyNDA5LjA3MDQudjEiLCJvcGVuYWxleCI6Ilc0NDAyNDY1NDE0IiwiZXhhIjoiaHR0cHM6Ly93d3cucHJlcHJpbnRzLm9yZy9tYW51c2NyaXB0LzIwMjQwOS4wNzA0In0sInRpdGxlIjoiQ29tcHJlaGVuc2l2ZSBSZXZpZXcgb2YgRXV0cm9waGljYXRpb24gaW4gRnJlc2h3YXRlciBFY29zeXN0ZW1zOiBDYXVzZXMsIEVmZmVjdHMsIEFzc2Vzc21lbnQsIGFuZCBNYW5hZ2VtZW50IFN0cmF0ZWdpZXMiLCJhYnN0cmFjdCI6IlRoaXMgcmV2aWV3IGV4YW1pbmVzIGV1dHJvcGhpY2F0aW9uIGluIGZyZXNod2F0ZXIgZWNvc3lzdGVtcywgYSB3aWRlc3ByZWFkIGVudmlyb25tZW50YWwgaXNzdWUgc3RlbW1pbmcgZnJvbSBleGNlc3NpdmUgbnV0cmllbnQgaW5wdXQsIGxhcmdlbHkgZHVlIHRvIGh1bWFuIGFjdGl2aXRpZXMuIFRoZSBzdHVkeSBleHBsb3JlcyB0aGUgb3JpZ2lucywgY29uc2VxdWVuY2VzLCBldmFsdWF0aW9uIHRlY2huaXF1ZXMsIGFuZCBjb250cm9sIG1ldGhvZHMgZm9yIGV1dHJvcGhpY2F0aW9uLiBQcmltYXJ5IGNhdXNlcyBpbmNsdWRlIGFncmljdWx0dXJhbCBkaXNjaGFyZ2UsIHVyYmFuIGRldmVsb3BtZW50LCBhbmQgY2xpbWF0ZSBzaGlmdHMuIFRoZSBlZmZlY3RzIGFyZSBmYXItcmVhY2hpbmcsIGltcGFjdGluZyBiaW9kaXZlcnNpdHksIHdhdGVyIHF1YWxpdHksIGFuZCBzb2Npby1lY29ub21pYyBmYWN0b3JzLiBUaGUgYXNzZXNzbWVudCB1dGlsaXplcyBhIGNvbWJpbmF0aW9uIG9mIGNoZW1pY2FsLCBwaHlzaWNhbCwgYW5kIGJpb2xvZ2ljYWwgbWFya2Vycywgd2l0aCBjdXR0aW5nLWVkZ2UgdGVjaG5vbG9naWVzIGxpa2UgcmVtb3RlIHNlbnNpbmcgaW1wcm92aW5nIG1vbml0b3JpbmcgZWZmb3J0cy4gTWl0aWdhdGlvbiBzdHJhdGVnaWVzIGFyZSBkaXZlcnNlLCBpbmNvcnBvcmF0aW5nIHByZXZlbnRhdGl2ZSBhY3Rpb25zIChzdWNoIGFzIG51dHJpZW50IGxpbWl0YXRpb24gYW5kIGxhbmQgbWFuYWdlbWVudCksIGNvcnJlY3RpdmUgbWVhc3VyZXMgKGluY2x1ZGluZyBiaW8tbWFuaXB1bGF0aW9uIGFuZCBjaGVtaWNhbCBhcHBsaWNhdGlvbnMpLCBhbmQgcmVndWxhdG9yeSBpbml0aWF0aXZlcyAobGlrZSB3YXRlciBxdWFsaXR5IGd1aWRlbGluZXMgYW5kIG51dHJpZW50IGV4Y2hhbmdlIHNjaGVtZXMpLiBXaGlsZSBjYXNlIHN0dWRpZXMgZGVtb25zdHJhdGUgc3VjY2Vzc2Z1bCBpbnRlcnZlbnRpb25zLCB0aGV5IGFsc28gcmV2ZWFsIG9uZ29pbmcgY2hhbGxlbmdlcywgcGFydGljdWxhcmx5IGVjb3N5c3RlbSB1bnByZWRpY3RhYmlsaXR5IGFuZCBjbGltYXRlIGNoYW5nZSBpbmZsdWVuY2VzLiBUaGUgcmVzZWFyY2ggaGlnaGxpZ2h0cyB0aGUgcHJvbWlzZSBvZiBlbWVyZ2luZyB0ZWNobm9sb2dpZXMsIG5vdGFibHkgbWFjaGluZSBsZWFybmluZywgaW4gZW5oYW5jaW5nIG1hbmFnZW1lbnQgcHJhY3RpY2VzLiBLbm93bGVkZ2UgZ2FwcyBwZXJzaXN0LCBlc3BlY2lhbGx5IHJlZ2FyZGluZyBldXRyb3BoaWNhdGlvbi1iaW9kaXZlcnNpdHkgZHluYW1pY3MgaW4gdHJvcGljYWwgZW52aXJvbm1lbnRzIGFuZCB0aGUgc29jaW8tZWNvbm9taWMgcmFtaWZpY2F0aW9ucyBvZiBpbnRlcnZlbnRpb25zLiBUaGUgc3R1ZHkgZW1waGFzaXplcyB0aGUgaW1wb3J0YW5jZSBvZiBmbGV4aWJsZSwgY29tcHJlaGVuc2l2ZSBhcHByb2FjaGVzIHRoYXQgYmxlbmQgc2NpZW50aWZpYyBpbnNpZ2h0cyB3aXRoIHByYWN0aWNhbCBzb2x1dGlvbnMgYW5kIHN0cm9uZyBwb2xpY3kgc3RydWN0dXJlcyB0byB0YWNrbGUgdGhpcyBpbnRyaWNhdGUgZW52aXJvbm1lbnRhbCBjaGFsbGVuZ2UgaW4gYW4gZXJhIG9mIGNsaW1hdGUgY2hhbmdlLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4yMDk0NC9wcmVwcmludHMyMDI0MDkuMDcwNC52MSIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cucHJlcHJpbnRzLm9yZy9mcm9udGVuZC9tYW51c2NyaXB0L2UwOGU1YjU2NTMzZTJhYWQ3NDdmZDk0MzVlNjdjYTczL2Rvd25sb2FkX3B1YiIsImlzc3VlZERhdGUiOiIyMDI0LTA5LTEwIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMiIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiUHJlcHJpbnRzLm9yZyIsImltcGFjdEZhY3RvciI6IjEuMTUifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjAsImNpdGVkQnlDb3VudCI6NX19LCJxdW90ZSI6IlRoZSBzdHVkeSBlbXBoYXNpemVzIHRoZSBpbXBvcnRhbmNlIG9mIGZsZXhpYmxlLCBjb21wcmVoZW5zaXZlIGFwcHJvYWNoZXMgdGhhdCBibGVuZCBzY2llbnRpZmljIGluc2lnaHRzIHdpdGggcHJhY3RpY2FsIHNvbHV0aW9ucyBhbmQgc3Ryb25nIHBvbGljeSBzdHJ1Y3R1cmVzIHRvIHRhY2tsZSB0aGlzIGludHJpY2F0ZSBlbnZpcm9ubWVudGFsIGNoYWxsZW5nZSBpbiBhbiBlcmEgb2YgY2xpbWF0ZSBjaGFuZ2UuIEV1dHJvcGhpY2F0aW9uLCBjYXVzZWQgYnkgZXhjZXNzaXZlIG51dHJpZW50IGVucmljaG1lbnQgaW4gd2F0ZXIgYm9kaWVzLCBpcyBhIHNpZ25pZmljYW50IGVudmlyb25tZW50YWwgY2hhbGxlbmdlIGZvciBmcmVzaHdhdGVyIGVjb3N5c3RlbXMgb24gYSBnbG9iYWwgc2NhbGUuIFByaW1hcmlseSBkcml2ZW4gYnkgbml0cm9nZW4gYW5kIHBob3NwaG9ydXMgaW5wdXRzLCB0aGlzIHByb2Nlc3MgcmVzdWx0cyBpbiB0aGUgb3ZlcnByb2R1Y3Rpb24gb2YgYWxnYWUgYW5kIGFxdWF0aWMgcGxhbnRzLCBsZWFkaW5nIHRvIGEgY2FzY2FkZSBvZiBlY29sb2dpY2FsIGRpc3R1cmJhbmNlcywgaW5jbHVkaW5nIG94eWdlbiBkZXBsZXRpb24gKGh5cG94aWEpLCBsb3NzIG9mIGJpb2RpdmVyc2l0eSwgYW5kIHNldmVyZSB3YXRlciBxdWFsaXR5IGRlZ3JhZGF0aW9uIChLYWthZGUgZXQgYWwuLCAyMDIxOyBNZWVyaG9mZiBldCBhbC4sIDIwMjI7IFd1cnRzYmF1Z2ggZXQgYWwuLCAyMDE5KS4ifV0%3D. Fish yields drop 40-70% in affected lakes like Chilika, where post-monsoon blooms recur annually (Nair & Nayak, 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hdnlhIFZpa3JhbWFuIiwiZmFtaWx5IjoiTmFpciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQcmF0ZWVwIEt1bWFyIiwiZmFtaWx5IjoiTmF5YWsifX1dLCJpZHMiOnsiaXNzbiI6WyIyMjk2LTc3NDUiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzODkvZm1hcnMuMjAyMy4xMDg3Mjk2Iiwib3BlbmFsZXgiOiJXNDM4MjcyMzEzMyJ9LCJ0aXRsZSI6IlVuY292ZXJpbmcgd2F0ZXIgcXVhbGl0eSBhbmQgZXZhbHVhdGluZyB2dWxuZXJhYmlsaXRpZXMgb2Ygc21hbGwtc2NhbGUgZmlzaGVyaWVzIGluIENoaWxpa2EgTGFnb29uLCBJbmRpYSIsImFic3RyYWN0IjoiU21hbGwtc2NhbGUgZmlzaGVyaWVzIChTU0ZzKSBwbGF5IGEgc3Ryb25nIHJvbGUgaW4gc3VzdGFpbmluZyBtaWxsaW9ucyBvZiBsaXZlbGlob29kcywgZm9vZCBzZWN1cml0eSwgbnV0cml0aW9uLCBhbmQgaW5jb21lIGdsb2JhbGx5IGJ1dCB0aGUgZmlzaGVycyBlbmdhZ2VkIGluIHRoaXMgc2VjdG9yIHNpbXVsdGFuZW91c2x5IGV4cGVyaWVuY2UgaGlnaCBsZXZlbHMgb2YgdnVsbmVyYWJpbGl0eSBhbmQgcHJvY2Vzc2VzIG9mIG1hcmdpbmFsaXNhdGlvbi4gU2V2ZXJhbCBmYWN0b3JzIGFyZSBhdHRyaWJ1dGVkIHRvIHRoZSBtdWx0aWRpbWVuc2lvbmFsIHZ1bG5lcmFiaWxpdGllcyB0aGUgc21hbGwtc2NhbGUgZmlzaGVycyBleHBlcmllbmNlLCBzcGFubmluZyBib3RoIG5hdHVyYWwgKGUuZy4sIG5hdHVyYWwgZGlzYXN0ZXJzLCBlY29zeXN0ZW0gY2hhbmdlKSBhbmQgYW50aHJvcG9nZW5pYyAoZS5nLiwgcG9saWN5IGNoYW5nZSwgaHlkcm9sb2dpY2FsIGludGVydmVudGlvbnMsIGFxdWFjdWx0dXJlKSBwcmVzc3VyZXMuIFdoaWxlIHRoZXJlIGlzIG11Y2ggbGl0ZXJhdHVyZSBvbiB2YXJpb3VzIG5hdHVyYWwgYW5kIGh1bWFuIGRyaXZlcnMgb2YgdnVsbmVyYWJpbGl0eSBpbiBzbWFsbC1zY2FsZSBmaXNoZXJ5IGNvbW11bml0aWVzLCBhbiBhYnNlbmNlIG9mIHJlc2VhcmNoIGNvbm5lY3RpbmcgdnVsbmVyYWJpbGl0eSB3aXRoIHdhdGVyIHF1YWxpdHkgaXMgZXZpZGVudC4gRmlzaGVyIGNvbW11bml0aWVzIG9mdGVuIHRhbGsgYWJvdXQgZmlzaCBpbiByZWxhdGlvbiB0byB0aGUgaGVhbHRoIG9mIHRoZWlyIGFxdWF0aWMgaGFiaXRhdHMgd2hlcmVpbiB3YXRlciBxdWFsaXR5IGlzIHNlZW4gYXMgYSBrZXkgcGFyYW1ldGVyLiBUaGUgbGluayBiZXR3ZWVuIGhlYWx0aHkgZmlzaCBhbmQgZ29vZCBxdWFsaXR5IHdhdGVyIGhhcyBzaWduaWZpY2FudCBpbXBsaWNhdGlvbnMgZm9yIHN0cm9uZyBhbmQgdmlhYmxlIGZpc2hpbmcgY29tbXVuaXRpZXMuIFRoaXMgcGFwZXIgZXhhbWluZXMgdGhlc2UgbGlua3MgZnVydGhlciBieSBmb2N1c2luZyBvbiB0aGUgbmF0dXJlIG9mIHZ1bG5lcmFiaWxpdGllcyBjYXVzZWQgYnkgd2F0ZXIgcXVhbGl0eSBjaGFuZ2VzIGluIHRoZSBzbWFsbC1zY2FsZSBmaXNoZXJ5IHN5c3RlbSBvZiBDaGlsaWthIExhZ29vbiBpbiBJbmRpYS4gV2UgdW5kZXJ0YWtlIGRldGFpbGVkIGFuYWx5c2lzIG9mIHRoZSBpbnZhc2l2ZSBzaHJpbXAgYXF1YWN1bHR1cmUgYWN0aXZpdGllcyBhbmQgaHlkcm9sb2dpY2FsIGludGVydmVudGlvbnMgZm9yIG9wZW5pbmcgb2YgYSBsYWdvb24gaW5sZXQgd2l0aCB0aGUgQmF5IG9mIEJlbmdhbCBhcyB0d28gZG9taW5hbnQgZHJpdmVycyBhZHZlcnNlbHkgaW1wYWN0aW5nIHdhdGVyIHF1YWxpdHkgYW5kIGluY3JlYXNpbmcgdnVsbmVyYWJpbGl0aWVzIG9mIHRoZSBlbnRpcmUgc21hbGwtc2NhbGUgZmlzaGVyaWVzIHNvY2lhbC1lY29sb2dpY2FsIHN5c3RlbS4gT3VyIGFuYWx5c2lzIHN1Z2dlc3RzIHRoYXQgdGhlcmUgYXJlIHN0cm9uZyBpbnRlcmNvbm5lY3Rpb25zIGJldHdlZW4gY2hhbmdlcyBpbiB3YXRlciBxdWFsaXR5IGFuZCB0aGUgbGV2ZWxzIG9mIHZ1bG5lcmFiaWxpdGllcyBpbiB0aGUgU1NGcyBvZiBDaGlsaWthIExhZ29vbi4gUG9sbHV0YW50cyBzdWNoIGFzIHBlc3RpY2lkZXMsIGFuZCBvcmdhbmljIGNvbXBvdW5kcyBhY2N1bXVsYXRlIGluIGZpc2ggdGlzc3VlcyBhbmQgYWZmZWN0IHRoZWlyIGdyb3d0aCwgcmVwcm9kdWN0aW9uLCBhbmQgb3ZlcmFsbCBoZWFsdGguIFRoaXMgbGVkIHRvIGRlY2xpbmVzIGluIGZpc2ggcG9wdWxhdGlvbnMsIG1ha2luZyBpdCBtb3JlIGRpZmZpY3VsdCBmb3IgZmlzaGVycyB0byBtYWtlIGEgbGl2aW5nLiBJbiBhZGRpdGlvbiB0byBkaXJlY3QgaW1wYWN0cyBvbiBmaXNoIHBvcHVsYXRpb25zLCBwb29yIHdhdGVyIHF1YWxpdHkgYWxzbyBoYXMgaW5kaXJlY3QgZWZmZWN0cyBvbiB0aGUgc29jaWFsIGFuZCBlY29ub21pYyB2dWxuZXJhYmlsaXR5IG9mIFNTRnMuIEZvciBleGFtcGxlLCBjb250YW1pbmF0aW9uIG9mIHdhdGVyIHNvdXJjZXMgbGVkIHRvIHRoZSByZWR1Y2VkIG51bWJlciBvZiBmaXNoIHNwZWNpZXMgcmVkdWNpbmcgdGhlIGFtb3VudCBvZiB0aW1lIGZpc2hlcnMgY2FuIHNwZW5kIG9uIGZpc2hpbmcgYWN0aXZpdGllcy4gVGhpcyBhbHNvIGFmZmVjdGVkIHRoZSBtYXJrZXRhYmlsaXR5IG9mIGZpc2ggcHJvZHVjdHMsIHJlZHVjZWQgaW5jb21lIGFuZCBpbmNyZWFzZWQgcG92ZXJ0eS4gVG8gZnVsbHkgdW5kZXJzdGFuZCB0aGUgaW50ZXJjb25uZWN0aW9ucyBiZXR3ZWVuIHdhdGVyIHF1YWxpdHkgYW5kIHZ1bG5lcmFiaWxpdHkgaW4gU1NGcyBpbiBDaGlsaWthIExhZ29vbiwgaXQgaXMgaW1wb3J0YW50IHRvIGNvbnNpZGVyIGJvdGggZW52aXJvbm1lbnRhbCBhbmQgc29jaWFsIGZhY3RvcnMsIGFzIHdlbGwgYXMgdGhlIGNvbXBsZXggZmVlZGJhY2sgbG9vcHMgYmV0d2VlbiB0aGVzZSBmYWN0b3JzLiBUaGUgc3R1ZHkgaGVscHMgaW4gYnJpZGdpbmcgYSBjcnVjaWFsIGdhcCBpbiBvdXIgdW5kZXJzdGFuZGluZyBvZiB0aGUgcm9sZSBvZiB3YXRlciBxdWFsaXR5IGluIHZ1bG5lcmFiaWxpdHkgYW5hbHlzaXMgd2l0aGluIHJlc291cmNlIGRlcGVuZGVudCBjb21tdW5pdGllcy4gV2UgY29uY2x1ZGUgd2l0aCBrZXkgaW5zaWdodHMgb24gcG9zc2libGUgY29waW5nIHJlc3BvbnNlcyBhbmQgYWRhcHRpdmUgY2FwYWNpdHkgbmVjZXNzYXJ5IGZvciB0aGUgc21hbGwtc2NhbGUgZmlzaGVyaWVzIGNvbW11bml0aWVzIHRvIHRyYW5zaXRpb24gdG93YXJkIHZpYWJpbGl0eS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMzM4OS9mbWFycy4yMDIzLjEwODcyOTYiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vd3d3LmZyb250aWVyc2luLm9yZy9hcnRpY2xlcy8xMC4zMzg5L2ZtYXJzLjIwMjMuMTA4NzI5Ni9wZGYiLCJpc3N1ZWREYXRlIjoiMjAyMy0wNi0yOSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZyb250aWVycyBpbiBNYXJpbmUgU2NpZW5jZSIsInZvbHVtZSI6MTAsImltcGFjdEZhY3RvciI6IjQuMjgifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRnJvbnRpZXJzIE1lZGlhIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo1MSwiY2l0ZWRCeUNvdW50Ijo1fX0sInF1b3RlIjoiV2F0ZXIgcXVhbGl0eSBvZiBDaGlsaWthIExhZ29vbiBoYXMgZGVncmFkZWQgZHVlIHRvIGFudGhyb3BvZ2VuaWMgaW50ZXJ2ZW50aW9ucyBhbmQgbmF0dXJhbCBmb3JjZXMgb2YgY2hhbmdlLiBWYXJpb3VzIGFjdGl2aXRpZXMgaW4gdGhlIENoaWxpa2EgTGFnb29uLCBzdWNoIGFzIGluZHVzdHJpYWwgd2FzdGV3YXRlciBkaXNwb3NhbCwgc2V3YWdlIGR1bXBpbmcsIGFxdWFjdWx0dXJlLCBoeWRyb2xvZ2ljYWwgaW50ZXJ2ZW50aW9ucywgYW5kIGN5Y2xvbmVzLCBhcmUgcHJvZHVjaW5nIHNhbGluaXR5IGNoYW5nZXMsIHNlZGltZW50IGRlcG9zaXRpb24sIG51dHJpZW50IGVucmljaG1lbnQsIGV1dHJvcGhpY2F0aW9uLCBhbmQgZGVhZCB6b25lcywgYXMgc3RhdGVkLiBGaXNoIHN1cnZpdmFsIGlzIGF0IHJpc2sgYXMgYSByZXN1bHQgb2YgdGhlIHNpZ25pZmljYW50IHJlZHVjdGlvbiBpbiBlbnZpcm9ubWVudGFsIGNpcmN1bXN0YW5jZXMsIHdoaWNoIGRpbWluaXNoZXMgZmlzaGVybWVu4oCZcyBpbmNvbWUgYW5kIHB1c2hlcyB0aGVtIG91dCBvZiBzb2NpZXR5LiBUaGVzZSBjYXVzZXMgYWZmZWN0aW5nIHRoZSB3YXRlciBxdWFsaXR5IG9mIHRoZSBMYWdvb24gZGVwaWN0IG1hbnkgZmFjZXMgb2YgdnVsbmVyYWJpbGl0eSBpbiBTU0YgY29tbXVuaXRpZXMuIEFzIHRoZSBDaGlsaWthIExhZ29vbiBlbnZpcm9ubWVudCBpcyBleHBlY3RlZCB0byB1bmRlcmdvIGluY3JlYXNpbmdseSBkcmFzdGljIGNoYW5nZXMgKHN1Y2ggYXMgY2F0Y2ggY29tcG9zaXRpb24sIGNhdGNoIGNhcGFjaXR5LCBiaW9kaXZlcnNpdHkgdmFyaWF0aW9ucywgd2F0ZXIgcXVhbGl0eSwgYW5kIGZpc2hlcnkgcmV2ZW51ZXMpLCBpbnRlcm5hdGlvbmFsIHBvbGljeSBoYXMgYmVjb21lIG1vcmUgaW50ZXJlc3RlZCBpbiBhZGFwdGF0aW9uIG1lYXN1cmVzLiBHaXZlbiBob3cgZGl2ZXJzZSBTU0YgZWNvc3lzdGVtcyBhcmUgZXhwZWN0ZWQgdG8gYmUgYmV0d2VlbiBjb250aW5lbnRzLCB0aGVyZSBpcyBubyDigJxvbmUgc2l6ZSBmaXRzIGFsbOKAnSBhcHByb2FjaC4gVGhlIHBhcGVyIG91dGxpbmVkIHNldmVyYWwgY29waW5nIGFuZCBhZGFwdGF0aW9uIG1lY2hhbmlzbXMgdG8gaW1wcm92ZSB3YXRlciBxdWFsaXR5IGFuZCBjb250aW51ZSBmaXNoaW5nIGluIGEgc3VzdGFpbmFibGUgd2F5LiBBZGFwdGF0aW9uIHN0cmF0ZWdpZXMgY2FuIG9jY2FzaW9uYWxseSBoYXZlIHVuZm9yZXNlZW4gY29uc2VxdWVuY2VzIGZvciBmaXNoZXJpZXMgYW5kIGZpc2hpbmcgY29tbXVuaXRpZXMuIEFueSB1bmV4cGVjdGVkIGNvbnNlcXVlbmNlcyBvZiBhZGFwdGF0aW9uIGNhbiBiZSBtaXRpZ2F0ZWQgdGhyb3VnaCBsb25nLXRlcm0gcGxhbm5pbmcgYW5kIGlkZW50aWZpY2F0aW9uIG9mIGZ1dHVyZSBvcHRpb25zIHRocm91Z2ggc2NlbmFyaW8gYW5hbHlzaXMuIFRoZSBmaW5kaW5ncyBvZiB0aGlzIHBhcGVyIGNhbiBiZSB1dGlsaXplZCB0byBkZXZlbG9wIHBvbGljaWVzIHRvIHByb3RlY3Qgd2F0ZXIgcXVhbGl0eSBhbmQgU1NGcyBpbiBhIHZhcmlldHkgb2YgY29hc3RhbCBsYWdvb24gc2V0dGluZ3MuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjIyLjI4NTAzNjA4NzAzNjEzMywibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ5MzA1MzQzNjI3OTMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjIzLjc3MzUxOTUxNTk5MTIxLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDgzMzY0MTA1MjI0NjEsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjI1LjI1NTU4NDcxNjc5Njg3NSwibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ4OTU1OTE3MzU4Mzk4NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjYuNzQ0MDY4MTQ1NzUxOTUzLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNTA2ODA1NDE5OTIxODc1LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyOC4yMjYxMzMzNDY1NTc2MTcsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40OTA0NjMyNTY4MzU5NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjkuNzE0NjE2Nzc1NTEyNjk1LCJsZWZ0Ijo4LjMzMTU5MjU1OTgxNDQ1Mywid2lkdGgiOjQwLjQ5NDA5ODY2MzMzMDA4LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozMS4xOTY2ODM4ODM2NjY5OTIsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40OTQzOTIzOTUwMTk1MywiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MzIuNjg1MTY1NDA1MjczNDQsImxlZnQiOjguMzMxNTk2Mzc0NTExNzE5LCJ3aWR0aCI6NDAuNDg3NDQ1ODMxMjk4ODMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM0LjE2NzIzMjUxMzQyNzczNCwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40OTc3NDU1MTM5MTYwMTYsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM1LjY1NTcxNTk0MjM4MjgxLCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ5ODYzODE1MzA3NjE3LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozNy4xMzc3ODMwNTA1MzcxMSwibGVmdCI6MTAuNjg1MTg2Mzg2MTA4Mzk4LCJ3aWR0aCI6MzguMTQxMTA1NjUxODU1NDcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM4LjYyNjI2NjQ3OTQ5MjE5LCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4NzE1MjA5OTYwOTM3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDAuMTA4MzMzNTg3NjQ2NDg0LCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4OTg1NjcxOTk3MDcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQxLjU5NjgxMzIwMTkwNDMsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDg4NjY2NTM0NDIzODMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQzLjA3ODg4MDMxMDA1ODU5NCwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40ODE4NjExMTQ1MDE5NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDQuNTY3MzYzNzM5MDEzNjcsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDgzMDU4OTI5NDQzMzYsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQ2LjA0OTQzMDg0NzE2Nzk3LCJsZWZ0Ijo4LjMzMzEwODkwMTk3NzUzOSwid2lkdGgiOjQwLjUwMDc0NzY4MDY2NDA2LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo0Ny41Mzc5MTQyNzYxMjMwNSwibGVmdCI6OC4zMzMxMDg5MDE5Nzc1MzksIndpZHRoIjo0MC40OTU2MDU0Njg3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDkuMDE5OTgxMzg0Mjc3MzQ0LCJsZWZ0Ijo4LjMzMzEwNjA0MDk1NDU5LCJ3aWR0aCI6NDAuNTA2ODE2ODY0MDEzNjcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUwLjUwODQ2NDgxMzIzMjQyLCJsZWZ0Ijo4LjMzMzEwNjA0MDk1NDU5LCJ3aWR0aCI6NDAuNDg4NTA2MzE3MTM4NjcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUxLjk5MDUyODEwNjY4OTQ1LCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDk3NzI2NDQwNDI5NjksImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUzLjQ3OTAxMTUzNTY0NDUzLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6MzMuMTU5NDQ2NzE2MzA4NTk0LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo1NC45NjEwNzg2NDM3OTg4MywibGVmdCI6MTAuNjgzNjcwMDQzOTQ1MzEyLCJ3aWR0aCI6MzguMTM2NTk2Njc5Njg3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NTYuNDQ5NTYyMDcyNzUzOTA2LCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDg3MTQ0NDcwMjE0ODQ0LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEzIn19XQ%3D%3D. Mitigation demands precision agriculture, riparian buffers, and monsoon-resilient practices, curbing fluxes to restore balance before 80% of lakes turn unusable (Kumar & Mahajan, 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlBhd2FuIiwiZmFtaWx5IjoiS3VtYXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW1icmlzaCBLdW1hciIsImZhbWlseSI6Ik1haGFqYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyNTIzLTM5NjMiLCIyNTIzLTM5NzEiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMDcvczQyNDUyLTAyMC0zMDgyLTgiLCJtYWciOiIzMDM1OTYwMjI4Iiwib3BlbmFsZXgiOiJXMzAzNTk2MDIyOCJ9LCJ0aXRsZSI6IlRyb3BoaWMgc3RhdHVzIGFuZCBpdHMgcmVndWxhdGluZyBmYWN0b3IgZGV0ZXJtaW5hdGlvbiBhdCB0aGUgUmV3YWxzYXIgTGFrZSwgbm9ydGh3ZXN0IEhpbWFsYXlhIChIUCksIEluZGlhLCBiYXNlZCBvbiBzZWxlY3RlZCBwYXJhbWV0ZXJzIGFuZCBtdWx0aXZhcmlhdGUgc3RhdGlzdGljYWwgYW5hbHlzaXMiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAwNy9zNDI0NTItMDIwLTMwODItOCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9saW5rLnNwcmluZ2VyLmNvbS9jb250ZW50L3BkZi8xMC4xMDA3L3M0MjQ1Mi0wMjAtMzA4Mi04LnBkZiIsImlzc3VlZERhdGUiOiIyMDIwLTA2LTIyIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiU04gQXBwbGllZCBTY2llbmNlcyIsImlzc3VlIjo3LCJ2b2x1bWUiOjIsImltcGFjdEZhY3RvciI6IjcuMzUifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiU3ByaW5nZXIgTmF0dXJlIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo0MCwiY2l0ZWRCeUNvdW50IjoxOX19LCJxdW90ZSI6IkNoYW5naW5nIHF1YWxpdHkgaGFzIGNyZWF0ZWQgdGhlIHdhdGVyIHNjYXJjaXR5IGFuZCAgYSBodWdlIG1hc3Mgb2YgcG9wdWxhdGlvbiB0aHVzIGJlZW4gZGVwcml2ZWQgZnJvbSB1c2luZyAgdGhlIGZyZXNod2F0ZXIgZm9yIHZhcmlvdXMgcHVycG9zZXMgWzMsIDRdLiBIb3dldmVyLCB0aGUgIG9ibm94aW91cyBtYXRlcmlhbHMgdGhhdCBzdGF5IGluIHdhdGVyIGJvZGllcyBmb3IgYSBsb25nLXRpbWUgcGVyaW9kIGJyaW5nIGNoYW5nZXMgaW4gdGhlIHBoeXNpb2NoZW1pY2FsIGNoYXItYWN0ZXJpc3RpY3MgYXMgd2VsbCBhcyB0aGUgbmF0dXJhbCBzY2VuYXJpbyBvZiB0aGUgbGFrZXMgWzVdLiAgVGhlIEhpbWFsYXlhbiBsYWtlcyBhcmUgdW5kZXIgc3RyZXNzIGR1ZSB0byBtaXNtYW5hZ2VkIGh1bWFuIGFjdGl2aXRpZXMsIHdoaWNoIGluY3JlYXNlIG51dHJpZW50cyBhbmQgcG9sbHV0aW9uIGluIHRlcm1zIG9mIGhlYXZ5IG1ldGFscyB0b3hpY2l0eSBhbmQgYmFjdGVyaWFsIGNvbnRhbWluYXRpb24gIFs24oCTOF0uIFRoZSBtYW5hZ2VtZW50IGFuZCBwcmVzZXJ2YXRpb24gb2YgdGhlIGxha2VzIGF0ICBwcmVzZW50IGhhdmUgYmVjb21lIGEgZ3JlYXQgY2hhbGxlbmdlIGZvciBlbnZpcm9ubWVudGFsLWlzdHMgYW5kIG1hbmFnZXJzIFs1LCA5XS4gVGhlIGV1dHJvcGhpY2F0aW9uIGlzIGFsc28gYSBtYWluIHByb2JsZW0gb2YgdGhlICBmcmVzaHdhdGVyIGxha2VzLCBhbmQgaXQgaXMgc3BlY2lmaWVkIGZvciB0aGUgbGFrZXMgaGF2aW5nICBudXRyaWVudCBlbnJpY2htZW50LiBJdCBjYXVzZXMgZXhjZXNzaXZlIGdyb3d0aCBvZiBhbGdhZSAgYW5kIGlzIGNvbnNpZGVyZWQgdG8gYmUgcmVzcG9uc2libGUgZm9yIGRlZ3JhZGluZyB0aGUgIHF1YWxpdGF0aXZlIG5hdHVyZSBvZiB0aGUgbGFrZXMgWzUsIDEwXS4gVGhlIGV1dHJvcGhpY2F0aW9uICBpcyBmb2xsb3dlZCBieSB0aGUgYWxnYWwgYmxvb20sIHdoaWNoIGdlbmVyYXRlcyB0b3hpbiwgIHdvcnNlbnMgd2F0ZXIgcXVhbGl0eSwgZGVwbGV0ZXMgZGlzc29sdmVkIG94eWdlbiAoRE8pICogUGF3YW4gS3VtYXIsIHBhd2FuLmV2c0BnbWFpbC5jb20gfCAxRGVwYXJ0bWVudCBvZiBFbnZpcm9ubWVudGFsIFNjaWVuY2VzLCBDZW50cmFsIFVuaXZlcnNpdHkgb2YgSGltYWNoYWwgUHJhZGVzaCwgRGhhcmFtc2hhbGEgMTc2MjE1LCBJbmRpYS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2OC41Nzg0NjA2OTMzNTkzOCwibGVmdCI6OC41NzE0MTg3NjIyMDcwMzEsIndpZHRoIjo0MC4zMjc1MTQ2NDg0Mzc1LCJoZWlnaHQiOjEuNTE5ODU1MjYwODQ4OTk5fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NzAuMTU4OTk2NTgyMDMxMjUsImxlZnQiOjguNTY5NzM4Mzg4MDYxNTIzLCJ3aWR0aCI6NDAuMzU0NzI0ODg0MDMzMiwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjcxLjczOTU0NzcyOTQ5MjE5LCJsZWZ0Ijo4LjU2OTczODM4ODA2MTUyMywid2lkdGgiOjQwLjM0OTY5MzI5ODMzOTg0NCwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjczLjMyMDA5MTI0NzU1ODYsImxlZnQiOjguNTY4MDU5OTIxMjY0NjQ4LCJ3aWR0aCI6NDAuMDIwMDg4MTk1ODAwNzgsImhlaWdodCI6MS41MTk4NTUyNjA4NDg5OTl9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo3NC45MDA2NDIzOTUwMTk1MywibGVmdCI6OC41NjgwNTk5MjEyNjQ2NDgsIndpZHRoIjozOS45OTk5Mjc1MjA3NTE5NSwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjc2LjQ4MTE4NTkxMzA4NTk0LCJsZWZ0Ijo4LjU2ODA1NzA2MDI0MTcsIndpZHRoIjo0MC4zNTcwNzQ3Mzc1NDg4MywiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjc4LjA2MTcyOTQzMTE1MjM0LCJsZWZ0Ijo4LjU2OTczNjQ4MDcxMjg5LCJ3aWR0aCI6NDAuMzUzNzEzOTg5MjU3ODEsImhlaWdodCI6MS41MTk4NTUyNjA4NDg5OTl9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2MC42NzU3Mjc4NDQyMzgyOCwibGVmdCI6NTEuNDI3MTY1OTg1MTA3NDIsIndpZHRoIjo0MC4zNzg1NjY3NDE5NDMzNiwiaGVpZ2h0IjoxOC45MDU4NTUxNzg4MzMwMDh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo4MS45MjczMzc2NDY0ODQzOCwibGVmdCI6OC40MDE0OTc4NDA4ODEzNDgsIndpZHRoIjo3My45NTQ2OTY2NTUyNzM0NCwiaGVpZ2h0IjoyLjY3ODc2MjY3NDMzMTY2NX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEifX1d. Untreated or inadequately treated wastewater constitutes a primary vector for nutrient loading, as urban centers channel substantial volumes of domestic sewage and industrial effluents directly into receiving water bodies (Sitote & Gebremedhine, 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllhYnNyYSBNZWxhayIsImZhbWlseSI6IlNpdG90ZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNaWNoYWVsIEdpcmltYXkiLCJmYW1pbHkiOiJHZWJyZW1lZGhpbmUifX1dLCJpZHMiOnsiaXNzbiI6W10sImlzYm4iOltdLCJkb2kiOiIxMC4yMDk0NC9wcmVwcmludHMyMDI0MDkuMDcwNC52MSIsIm9wZW5hbGV4IjoiVzQ0MDI0NjU0MTQiLCJleGEiOiJodHRwczovL3d3dy5wcmVwcmludHMub3JnL21hbnVzY3JpcHQvMjAyNDA5LjA3MDQifSwidGl0bGUiOiJDb21wcmVoZW5zaXZlIFJldmlldyBvZiBFdXRyb3BoaWNhdGlvbiBpbiBGcmVzaHdhdGVyIEVjb3N5c3RlbXM6IENhdXNlcywgRWZmZWN0cywgQXNzZXNzbWVudCwgYW5kIE1hbmFnZW1lbnQgU3RyYXRlZ2llcyIsImFic3RyYWN0IjoiVGhpcyByZXZpZXcgZXhhbWluZXMgZXV0cm9waGljYXRpb24gaW4gZnJlc2h3YXRlciBlY29zeXN0ZW1zLCBhIHdpZGVzcHJlYWQgZW52aXJvbm1lbnRhbCBpc3N1ZSBzdGVtbWluZyBmcm9tIGV4Y2Vzc2l2ZSBudXRyaWVudCBpbnB1dCwgbGFyZ2VseSBkdWUgdG8gaHVtYW4gYWN0aXZpdGllcy4gVGhlIHN0dWR5IGV4cGxvcmVzIHRoZSBvcmlnaW5zLCBjb25zZXF1ZW5jZXMsIGV2YWx1YXRpb24gdGVjaG5pcXVlcywgYW5kIGNvbnRyb2wgbWV0aG9kcyBmb3IgZXV0cm9waGljYXRpb24uIFByaW1hcnkgY2F1c2VzIGluY2x1ZGUgYWdyaWN1bHR1cmFsIGRpc2NoYXJnZSwgdXJiYW4gZGV2ZWxvcG1lbnQsIGFuZCBjbGltYXRlIHNoaWZ0cy4gVGhlIGVmZmVjdHMgYXJlIGZhci1yZWFjaGluZywgaW1wYWN0aW5nIGJpb2RpdmVyc2l0eSwgd2F0ZXIgcXVhbGl0eSwgYW5kIHNvY2lvLWVjb25vbWljIGZhY3RvcnMuIFRoZSBhc3Nlc3NtZW50IHV0aWxpemVzIGEgY29tYmluYXRpb24gb2YgY2hlbWljYWwsIHBoeXNpY2FsLCBhbmQgYmlvbG9naWNhbCBtYXJrZXJzLCB3aXRoIGN1dHRpbmctZWRnZSB0ZWNobm9sb2dpZXMgbGlrZSByZW1vdGUgc2Vuc2luZyBpbXByb3ZpbmcgbW9uaXRvcmluZyBlZmZvcnRzLiBNaXRpZ2F0aW9uIHN0cmF0ZWdpZXMgYXJlIGRpdmVyc2UsIGluY29ycG9yYXRpbmcgcHJldmVudGF0aXZlIGFjdGlvbnMgKHN1Y2ggYXMgbnV0cmllbnQgbGltaXRhdGlvbiBhbmQgbGFuZCBtYW5hZ2VtZW50KSwgY29ycmVjdGl2ZSBtZWFzdXJlcyAoaW5jbHVkaW5nIGJpby1tYW5pcHVsYXRpb24gYW5kIGNoZW1pY2FsIGFwcGxpY2F0aW9ucyksIGFuZCByZWd1bGF0b3J5IGluaXRpYXRpdmVzIChsaWtlIHdhdGVyIHF1YWxpdHkgZ3VpZGVsaW5lcyBhbmQgbnV0cmllbnQgZXhjaGFuZ2Ugc2NoZW1lcykuIFdoaWxlIGNhc2Ugc3R1ZGllcyBkZW1vbnN0cmF0ZSBzdWNjZXNzZnVsIGludGVydmVudGlvbnMsIHRoZXkgYWxzbyByZXZlYWwgb25nb2luZyBjaGFsbGVuZ2VzLCBwYXJ0aWN1bGFybHkgZWNvc3lzdGVtIHVucHJlZGljdGFiaWxpdHkgYW5kIGNsaW1hdGUgY2hhbmdlIGluZmx1ZW5jZXMuIFRoZSByZXNlYXJjaCBoaWdobGlnaHRzIHRoZSBwcm9taXNlIG9mIGVtZXJnaW5nIHRlY2hub2xvZ2llcywgbm90YWJseSBtYWNoaW5lIGxlYXJuaW5nLCBpbiBlbmhhbmNpbmcgbWFuYWdlbWVudCBwcmFjdGljZXMuIEtub3dsZWRnZSBnYXBzIHBlcnNpc3QsIGVzcGVjaWFsbHkgcmVnYXJkaW5nIGV1dHJvcGhpY2F0aW9uLWJpb2RpdmVyc2l0eSBkeW5hbWljcyBpbiB0cm9waWNhbCBlbnZpcm9ubWVudHMgYW5kIHRoZSBzb2Npby1lY29ub21pYyByYW1pZmljYXRpb25zIG9mIGludGVydmVudGlvbnMuIFRoZSBzdHVkeSBlbXBoYXNpemVzIHRoZSBpbXBvcnRhbmNlIG9mIGZsZXhpYmxlLCBjb21wcmVoZW5zaXZlIGFwcHJvYWNoZXMgdGhhdCBibGVuZCBzY2llbnRpZmljIGluc2lnaHRzIHdpdGggcHJhY3RpY2FsIHNvbHV0aW9ucyBhbmQgc3Ryb25nIHBvbGljeSBzdHJ1Y3R1cmVzIHRvIHRhY2tsZSB0aGlzIGludHJpY2F0ZSBlbnZpcm9ubWVudGFsIGNoYWxsZW5nZSBpbiBhbiBlcmEgb2YgY2xpbWF0ZSBjaGFuZ2UuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjIwOTQ0L3ByZXByaW50czIwMjQwOS4wNzA0LnYxIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjA5NDQvcHJlcHJpbnRzMjAyNDA5LjA3MDQudjEiLCJpc3N1ZWREYXRlIjoiMjAyNC0wOS0xMCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTEiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IlByZXByaW50cy5vcmciLCJpbXBhY3RGYWN0b3IiOiIxLjE1In0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjowLCJjaXRlZEJ5Q291bnQiOjR9fSwicXVvdGUiOiJFdXRyb3BoaWNhdGlvbiBpbiBmcmVzaHdhdGVyIHN5c3RlbXMgaXMgcHJpbWFyaWx5IGRyaXZlbiBieSB0aGUgZXhjZXNzaXZlIGlucHV0IG9mIG51dHJpZW50cywgcGFydGljdWxhcmx5IG5pdHJvZ2VuIChOKSBhbmQgcGhvc3Bob3J1cyAoUCkuIFRoZXNlIG51dHJpZW50cyBvZnRlbiBlbnRlciB3YXRlciBib2RpZXMgdGhyb3VnaCBhZ3JpY3VsdHVyYWwgcnVub2ZmLCB3aGVyZSBmZXJ0aWxpemVycywgbGl2ZXN0b2NrIHdhc3RlLCBhbmQgc29pbCBlcm9zaW9uIGFyZSBzaWduaWZpY2FudCBjb250cmlidXRvcnMgKEdyaWZmaXRoICYgR29ibGVyLCAyMDIwKS4gVXJiYW4gYXJlYXMgYWxzbyBjb250cmlidXRlIHRvIG51dHJpZW50IGxvYWRpbmcgdGhyb3VnaCB1bnRyZWF0ZWQgb3IgaW5hZGVxdWF0ZWx5IHRyZWF0ZWQgd2FzdGV3YXRlciwgc3Rvcm13YXRlciBydW5vZmYsIGFuZCBpbmR1c3RyaWFsIGVmZmx1ZW50cywgYWxsIG9mIHdoaWNoIGNhcnJ5IHN1YnN0YW50aWFsIGFtb3VudHMgb2YgbnV0cmllbnRzIGludG8gcml2ZXJzLCBsYWtlcywgYW5kIG90aGVyIGZyZXNod2F0ZXIgYm9kaWVzIChXdXJ0c2JhdWdoIGV0IGFsLiwgMjAxOSkuIExhbmQgdXNlIGNoYW5nZXMsIGluY2x1ZGluZyB1cmJhbml6YXRpb24sIGRlZm9yZXN0YXRpb24sIGFuZCBhZ3JpY3VsdHVyYWwgZXhwYW5zaW9uLCBoYXZlIHNpZ25pZmljYW50bHkgZGlzcnVwdGVkIG5hdHVyYWwgbnV0cmllbnQgY3ljbGVzLiIsImV4dGVybmFsQ2FjaGVJZCI6ImVlN2FiZWEzLWYwYmItMzU4Ni05Y2MzLTc4NTlkZWI1YzZjZCJ9XQ%3D%3D. The resulting influx of organic matter and ammonium significantly alters the intensity and toxicity of harmful cyanobacterial blooms, particularly in systems where hydraulic retention times are short and dilution capacity is limited (Gobler et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkNocmlzdG9waGVyIEouIiwiZmFtaWx5IjoiR29ibGVyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlJ1dGggVy4iLCJmYW1pbHkiOiJEcmlua3dhdGVyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkEuIFcuIiwiZmFtaWx5IjoiQW50aG9ueSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJKZW5uaWZlciBBLiIsImZhbWlseSI6IkdvbGVza2kifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW5uIE1hcmllIEUuIiwiZmFtaWx5IjoiRmFtdWxhcm8tUGVjb3JhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1pY2hhZWwgQi4iLCJmYW1pbHkiOiJXYWxsYWNlIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5vcmEgUi4gVy4iLCJmYW1pbHkiOiJTdHJhcXVhZGluZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJSb25vam95IiwiZmFtaWx5IjoiSGVtIn19XSwiaWRzIjp7Imlzc24iOlsiMTY2NC0zMDJYIl0sImlzYm4iOltdLCJkb2kiOiIxMC4zMzg5L2ZtaWNiLjIwMjQuMTQ2NDY4NiIsInBtaWQiOiIzOTU2ODk5NyIsIm9wZW5hbGV4IjoiVzQ0MDQxMjA5NTIiLCJleGEiOiJodHRwczovL3d3dy5mcm9udGllcnNpbi5vcmcvam91cm5hbHMvbWljcm9iaW9sb2d5L2FydGljbGVzLzEwLjMzODkvZm1pY2IuMjAyNC4xNDY0Njg2L2Z1bGwifSwidGl0bGUiOiJTZXdhZ2UtYW5kIGZlcnRpbGl6ZXItZGVyaXZlZCBudXRyaWVudHMgYWx0ZXIgdGhlIGludGVuc2l0eSwgZGl2ZXJzaXR5LCBhbmQgdG94aWNpdHkgb2YgaGFybWZ1bCBjeWFub2JhY3RlcmlhbCBibG9vbXMgaW4gZXV0cm9waGljIGxha2VzIiwiYWJzdHJhY3QiOiJDeWFub2JhY3RlcmlhbCBoYXJtZnVsIGFsZ2FsIGJsb29tcyAoQ0hBQnMpIGFyZSBwcm9tb3RlZCBieSBleGNlc3NpdmUgbnV0cmllbnQgbG9hZGluZyBhbmQsIHdoaWxlIGZlcnRpbGl6ZXJzIGFuZCBzZXdhZ2UgYXJlIHRoZSBtb3N0IHByZXZhbGVudCBleHRlcm5hbCBudXRyaWVudCBzb3VyY2VzIGluIG1vc3Qgd2F0ZXJzaGVkcywgdGhlIGRpZmZlcmVudGlhbCBlZmZlY3RzIG9mIHRoZXNlIG51dHJpZW50IHNvdXJjZXMgb24gQ0hBQnMgYXJlIHVua25vd24uIEhlcmUsIHdlIHRyYWNrZWQgQ0hBQnMgYW5kIHBlcmZvcm1lZCBleHBlcmltZW50cyBpbiBmaXZlIGRpc3RpbmN0IGxha2VzIGFjcm9zcyB0aGUgTm9ydGhlcm4gVVMgaW5jbHVkaW5nIExha2UgRXJpZS4gRmVydGlsaXplcnMgd2l0aCBhbW1vbml1bSBhbmQgb3J0aG9waG9zcGhhdGUsIG1lbWJyYW5lICgwLjIgzrxtKS1maWx0ZXJlZCBzZXdhZ2UgKGRvbWluYXRlZCBieSByZWR1Y2VkIGZvcm1zIG9mIG5pdHJvZ2VuKSBzYW5kLWFuZCBtZW1icmFuZS1maWx0ZXJlZCBzZXdhZ2UgKGRvbWluYXRlZCBieSBuaXRyYXRlKSwgYW5kIGFuIGlub3JnYW5pYyBudXRyaWVudCBzb2x1dGlvbiBvZiBhbW1vbml1bSBhbmQgb3J0aG9waG9zcGhhdGUgd2VyZSB1c2VkIGFzIGV4cGVyaW1lbnRhbCBudXRyaWVudCBzb3VyY2VzIGZvciBDSEFCcyBhdCBOLWVxdWl2YWxlbnQsIGVudmlyb25tZW50YWxseSByZWFsaXN0aWMgY29uY2VudHJhdGlvbnMuIFBoeXRvcGxhbmt0b24gY29tbXVuaXRpZXMgd2VyZSBldmFsdWF0ZWQgZmx1b3JvbWV0cmljYWxseSwgbWljcm9zY29waWNhbGx5LCBhbmQgdmlhIGhpZ2ggdGhyb3VnaHB1dCBzZXF1ZW5jaW5nIG9mIHRoZSAxNlMgclJOQSBnZW5lLCBhbmQgbGV2ZWxzIG9mIG1pY3JvY3lzdGluIGFuZCB0aGUgzrQgMTUgTiBjb250ZW50IG9mIHBhcnRpY3VsYXRlIG9yZ2FuaWMgbml0cm9nZW4gKM60UE8gMTUgTikgd2VyZSBxdWFudGlmaWVkLiBGZXJ0aWxpemVyIGFuZCBib3RoIHNvdXJjZXMgb2Ygd2FzdGV3YXRlciBpbmNyZWFzZWQgdGhlIGFidW5kYW5jZSBvZiBjeWFub2JhY3RlcmlhIGluIGFsbCBleHBlcmltZW50cyBhY3Jvc3MgYWxsIGZpdmUgbGFrZXMgKCBwICZhbXA7bHQ7IDAuMDUgZm9yIGFsbCkgd2hlcmVhcyBlZmZlY3RzIG9uIGV1a2FyeW90aWMgcGh5dG9wbGFua3RvbiB3ZXJlIGxpbWl0ZWQuIFNhbmQtZmlsdGVyZWQgc2V3YWdlIGNvbnRhaW5lZCBsZXNzIFAsIG9yZ2FuaWMgbWF0dGVyLCBhbmQgYW1tb25pdW0gYnV0IG1vcmUgbml0cmF0ZSBhbmQgaGFkIGEgMjUlIGxlc3MgcG90ZW50IHN0aW11bGF0b3J5IGVmZmVjdCBvbiBjeWFub2JhY3RlcmlhIHRoYW4gbWVtYnJhbmUtZmlsdGVyZWQgc2V3YWdlLCBzdWdnZXN0aW5nIG5pdHJpZmljYXRpb24gbWF5IHBsYXkgYSByb2xlIGluIHJlZHVjaW5nIENIQUJzLiBGZXJ0aWxpemVyIGluY3JlYXNlZCBtaWNyb2N5c3RpbiBsZXZlbHMgYW5kIGRlY3JlYXNlZCB0aGUgzrRQTyAxNSBOIHdoZXJlYXMgd2FzdGV3YXRlciBpbmNyZWFzZWQgzrRQTyAxNSBOICggcCAmYW1wO2x0OyAwLjA1IGZvciBhbGwpLiBNaWNyb2N5c3RpcyB3YXMgdGhlIGdlbnVzIG1vc3QgY29uc2lzdGVudGx5IHByb21vdGVkIGJ5IG51dHJpZW50IHNvdXJjZXMgKCBwICZhbXA7bHQ7IDAuMDUgaW4gYWxsIGV4cGVyaW1lbnRzKSwgZm9sbG93ZWQgYnkgQ3lhbm9iaXVtICggcCAmYW1wO2x0OyAwLjA1IGluIDUwJSBvZiBleHBlcmltZW50cyksIHdpdGggaW5jcmVhc2VzIGluIE1pY3JvY3lzdGlzIGJpb21hc3MgY29uc2lzdGVudGx5IGVsaWNpdGVkIGJ5IG1lbWJyYW5lLWZpbHRlcmVkIHdhc3Rld2F0ZXIuIENvbGxlY3RpdmVseSwgcmVzdWx0cyBkZW1vbnN0cmF0ZSB0aGF0IGRpZmZlcmluZyB0eXBlcyBvZiBzZXdhZ2UgZGlzY2hhcmdlIGFuZCBmZXJ0aWxpemVycyBjYW4gcHJvbW90ZSBDSEFCIGludGVuc2l0eSBhbmQgdG94aWNpdHksIHdoaWxlIGNvbmN1cnJlbnRseSBhbHRlcmluZyBDSEFCIGRpdmVyc2l0eSBhbmQgzrRQTyAxNSBOLiBXaGlsZSBtZW1icmFuZS1maWx0ZXJlZCBzZXdhZ2UgY29uc2lzdGVudGx5IGZhdm9yZWQgTWljcm9jeXN0aXMgLCB0aGUgZGlzY2hhcmdlIG9mIHNld2FnZSB0aHJvdWdoIHNhbmRzIG11dGVkIGJsb29tIGludGVuc2l0eSBzdWdnZXN0aW5nIHNhbmQtYmVkcyBtYXkgcmVwcmVzZW50IGEgdG9vbCB0byByZW1vdmUga2V5IG51dHJpZW50cyBhbmQgcGFydGlhbGx5IG1pdGlnYXRlIENIQUJzLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzg5L2ZtaWNiLjIwMjQuMTQ2NDY4NiIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cuZnJvbnRpZXJzaW4ub3JnL2pvdXJuYWxzL21pY3JvYmlvbG9neS9hcnRpY2xlcy8xMC4zMzg5L2ZtaWNiLjIwMjQuMTQ2NDY4Ni9wZGYiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMS0wNiIsImFjY2Vzc2VkRGF0ZSI6IjIwMjYtMDIiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZyb250aWVycyBpbiBNaWNyb2Jpb2xvZ3kiLCJ2b2x1bWUiOjE1LCJpbXBhY3RGYWN0b3IiOiIzLjk5In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTQ2NDY4NiwibGFzdFBhZ2UiOjE0NjQ2ODZ9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRnJvbnRpZXJzIE1lZGlhIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo3MywiY2l0ZWRCeUNvdW50Ijo5fX0sInF1b3RlIjoiZGVyaXZlZCBudXRyaWVudHMgYWx0ZXIgdGhlIGludGVuc2l0eSwgZGl2ZXJzaXR5LCBhbmQgdG94aWNpdHkgb2YgaGFybWZ1bCBjeWFub2JhY3RlcmlhbCBibG9vbXMgLi4uIEN5YW5vYmFjdGVyaWFsIGhhcm1mdWwgYWxnYWwgYmxvb21zIChDSEFCcykgYXJlIHByb21vdGVkIGJ5IGV4Y2Vzc2l2ZSBudXRyaWVudCBsb2FkaW5nIGFuZCwgd2hpbGUgZmVydGlsaXplcnMgYW5kIHNld2FnZSBhcmUgdGhlIG1vc3QgcHJldmFsZW50IGV4dGVybmFsIG51dHJpZW50IHNvdXJjZXMgaW4gbW9zdCB3YXRlcnNoZWRzLCB0aGUgZGlmZmVyZW50aWFsIGVmZmVjdHMgb2YgdGhlc2UgbnV0cmllbnQgLi4uIG9yZ2FuaWMgbml0cm9nZW4gKM60UE8xNU4pIHdlcmUgcXVhbnRpZmllZC4gRmVydGlsaXplciBhbmQgYm90aCBzb3VyY2VzIG9mIHdhc3Rld2F0ZXIgaW5jcmVhc2VkIHRoZSBhYnVuZGFuY2Ugb2YgLi4uIHBoeXRvcGxhbmt0b24gd2VyZSBsaW1pdGVkLiBTYW5kLWZpbHRlcmVkIHNld2FnZSBjb250YWluZWQgbGVzcyBQLCBvcmdhbmljIG1hdHRlciwgYW5kIGFtbW9uaXVtIGJ1dCBtb3JlIG5pdHJhdGUgYW5kIC4uLiBkaWZmZXJpbmcgdHlwZXMgb2Ygc2V3YWdlIGRpc2NoYXJnZSBhbmQgZmVydGlsaXplcnMgY2FuIHByb21vdGUgQ0hBQiBpbnRlbnNpdHkgYW5kIHRveGljaXR5LCB3aGlsZSBjb25jdXJyZW50bHkgLi4uIHNld2FnZSB0aHJvdWdoIHNhbmRzIG11dGVkIGJsb29tIGludGVuc2l0eSBzdWdnZXN0aW5nIHNhbmQtYmVkcyBtYXkgcmVwcmVzZW50IGEgdG9vbCB0byByZW1vdmUga2V5IG51dHJpZW50cyBhbmQgLi4uICggV2F0c29uIGV0IGFsLiwgMjAxNiA7IEJ1bGxlcmphaG4gZXQgYWwuLCAyMDE2IDsgSHVpc21hbiBldCBhbC4sIDIwMTggKSBhbmQgZXhjZXNzaXZlIG51dHJpZW50IGxvYWRpbmcgaXMgdGhlIHByb2Nlc3MgbW9zdCBjb21tb25seSBjaXRlZCBhcyB0aGUgZW52aXJvbm1lbnRhbCBmYWN0b3IgcHJvbW90aW5nIENIQUJzICgiLCJleHRlcm5hbENhY2hlSWQiOiJkYzMyMDY5MC03YjljLTMwOGEtYjJkMy0xYzA3MjVkNTUyNjgifV0%3D.

5.5. Urban Sewage and Industrial Discharge: The Infrastructure Gap

Urban sewage and industrial effluents represent a critical driver of eutrophication in India's peri-urban and urban lakes, where inadequate infrastructure allows untreated wastewater to flow directly into these vital water bodies. With over 70% of India's urban sewage discharged untreated due to insufficient treatment capacity estimated at a mere 30% coverage lakes receive massive nutrient loads from domestic sources rich in nitrogen (as ammonium) and phosphorus (from detergents), alongside organic matter (Kumar & Mahajan, 2020; Sitote & Gebremedhine, 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlBhd2FuIiwiZmFtaWx5IjoiS3VtYXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW1icmlzaCBLdW1hciIsImZhbWlseSI6Ik1haGFqYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyNTIzLTM5NjMiLCIyNTIzLTM5NzEiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMDcvczQyNDUyLTAyMC0zMDgyLTgiLCJtYWciOiIzMDM1OTYwMjI4Iiwib3BlbmFsZXgiOiJXMzAzNTk2MDIyOCJ9LCJ0aXRsZSI6IlRyb3BoaWMgc3RhdHVzIGFuZCBpdHMgcmVndWxhdGluZyBmYWN0b3IgZGV0ZXJtaW5hdGlvbiBhdCB0aGUgUmV3YWxzYXIgTGFrZSwgbm9ydGh3ZXN0IEhpbWFsYXlhIChIUCksIEluZGlhLCBiYXNlZCBvbiBzZWxlY3RlZCBwYXJhbWV0ZXJzIGFuZCBtdWx0aXZhcmlhdGUgc3RhdGlzdGljYWwgYW5hbHlzaXMiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAwNy9zNDI0NTItMDIwLTMwODItOCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9saW5rLnNwcmluZ2VyLmNvbS9jb250ZW50L3BkZi8xMC4xMDA3L3M0MjQ1Mi0wMjAtMzA4Mi04LnBkZiIsImlzc3VlZERhdGUiOiIyMDIwLTA2LTIyIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiU04gQXBwbGllZCBTY2llbmNlcyIsImlzc3VlIjo3LCJ2b2x1bWUiOjIsImltcGFjdEZhY3RvciI6IjcuMzUifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiU3ByaW5nZXIgTmF0dXJlIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo0MCwiY2l0ZWRCeUNvdW50IjoxOX19LCJxdW90ZSI6IkNoYW5naW5nIHF1YWxpdHkgaGFzIGNyZWF0ZWQgdGhlIHdhdGVyIHNjYXJjaXR5IGFuZCAgYSBodWdlIG1hc3Mgb2YgcG9wdWxhdGlvbiB0aHVzIGJlZW4gZGVwcml2ZWQgZnJvbSB1c2luZyAgdGhlIGZyZXNod2F0ZXIgZm9yIHZhcmlvdXMgcHVycG9zZXMgWzMsIDRdLiBIb3dldmVyLCB0aGUgIG9ibm94aW91cyBtYXRlcmlhbHMgdGhhdCBzdGF5IGluIHdhdGVyIGJvZGllcyBmb3IgYSBsb25nLXRpbWUgcGVyaW9kIGJyaW5nIGNoYW5nZXMgaW4gdGhlIHBoeXNpb2NoZW1pY2FsIGNoYXItYWN0ZXJpc3RpY3MgYXMgd2VsbCBhcyB0aGUgbmF0dXJhbCBzY2VuYXJpbyBvZiB0aGUgbGFrZXMgWzVdLiAgVGhlIEhpbWFsYXlhbiBsYWtlcyBhcmUgdW5kZXIgc3RyZXNzIGR1ZSB0byBtaXNtYW5hZ2VkIGh1bWFuIGFjdGl2aXRpZXMsIHdoaWNoIGluY3JlYXNlIG51dHJpZW50cyBhbmQgcG9sbHV0aW9uIGluIHRlcm1zIG9mIGhlYXZ5IG1ldGFscyB0b3hpY2l0eSBhbmQgYmFjdGVyaWFsIGNvbnRhbWluYXRpb24gIFs24oCTOF0uIFRoZSBtYW5hZ2VtZW50IGFuZCBwcmVzZXJ2YXRpb24gb2YgdGhlIGxha2VzIGF0ICBwcmVzZW50IGhhdmUgYmVjb21lIGEgZ3JlYXQgY2hhbGxlbmdlIGZvciBlbnZpcm9ubWVudGFsLWlzdHMgYW5kIG1hbmFnZXJzIFs1LCA5XS4gVGhlIGV1dHJvcGhpY2F0aW9uIGlzIGFsc28gYSBtYWluIHByb2JsZW0gb2YgdGhlICBmcmVzaHdhdGVyIGxha2VzLCBhbmQgaXQgaXMgc3BlY2lmaWVkIGZvciB0aGUgbGFrZXMgaGF2aW5nICBudXRyaWVudCBlbnJpY2htZW50LiBJdCBjYXVzZXMgZXhjZXNzaXZlIGdyb3d0aCBvZiBhbGdhZSAgYW5kIGlzIGNvbnNpZGVyZWQgdG8gYmUgcmVzcG9uc2libGUgZm9yIGRlZ3JhZGluZyB0aGUgIHF1YWxpdGF0aXZlIG5hdHVyZSBvZiB0aGUgbGFrZXMgWzUsIDEwXS4gVGhlIGV1dHJvcGhpY2F0aW9uICBpcyBmb2xsb3dlZCBieSB0aGUgYWxnYWwgYmxvb20sIHdoaWNoIGdlbmVyYXRlcyB0b3hpbiwgIHdvcnNlbnMgd2F0ZXIgcXVhbGl0eSwgZGVwbGV0ZXMgZGlzc29sdmVkIG94eWdlbiAoRE8pICogUGF3YW4gS3VtYXIsIHBhd2FuLmV2c0BnbWFpbC5jb20gfCAxRGVwYXJ0bWVudCBvZiBFbnZpcm9ubWVudGFsIFNjaWVuY2VzLCBDZW50cmFsIFVuaXZlcnNpdHkgb2YgSGltYWNoYWwgUHJhZGVzaCwgRGhhcmFtc2hhbGEgMTc2MjE1LCBJbmRpYS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2OC41Nzg0NjA2OTMzNTkzOCwibGVmdCI6OC41NzE0MTg3NjIyMDcwMzEsIndpZHRoIjo0MC4zMjc1MTQ2NDg0Mzc1LCJoZWlnaHQiOjEuNTE5ODU1MjYwODQ4OTk5fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NzAuMTU4OTk2NTgyMDMxMjUsImxlZnQiOjguNTY5NzM4Mzg4MDYxNTIzLCJ3aWR0aCI6NDAuMzU0NzI0ODg0MDMzMiwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjcxLjczOTU0NzcyOTQ5MjE5LCJsZWZ0Ijo4LjU2OTczODM4ODA2MTUyMywid2lkdGgiOjQwLjM0OTY5MzI5ODMzOTg0NCwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjczLjMyMDA5MTI0NzU1ODYsImxlZnQiOjguNTY4MDU5OTIxMjY0NjQ4LCJ3aWR0aCI6NDAuMDIwMDg4MTk1ODAwNzgsImhlaWdodCI6MS41MTk4NTUyNjA4NDg5OTl9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo3NC45MDA2NDIzOTUwMTk1MywibGVmdCI6OC41NjgwNTk5MjEyNjQ2NDgsIndpZHRoIjozOS45OTk5Mjc1MjA3NTE5NSwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjc2LjQ4MTE4NTkxMzA4NTk0LCJsZWZ0Ijo4LjU2ODA1NzA2MDI0MTcsIndpZHRoIjo0MC4zNTcwNzQ3Mzc1NDg4MywiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjc4LjA2MTcyOTQzMTE1MjM0LCJsZWZ0Ijo4LjU2OTczNjQ4MDcxMjg5LCJ3aWR0aCI6NDAuMzUzNzEzOTg5MjU3ODEsImhlaWdodCI6MS41MTk4NTUyNjA4NDg5OTl9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2MC42NzU3Mjc4NDQyMzgyOCwibGVmdCI6NTEuNDI3MTY1OTg1MTA3NDIsIndpZHRoIjo0MC4zNzg1NjY3NDE5NDMzNiwiaGVpZ2h0IjoxOC45MDU4NTUxNzg4MzMwMDh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo4MS45MjczMzc2NDY0ODQzOCwibGVmdCI6OC40MDE0OTc4NDA4ODEzNDgsIndpZHRoIjo3My45NTQ2OTY2NTUyNzM0NCwiaGVpZ2h0IjoyLjY3ODc2MjY3NDMzMTY2NX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEifX0seyJvcmlnaW4iOjUsIndvcmsiOnsidHlwZSI6OCwic3VidHlwZSI6eyJvbmVvZktpbmQiOiJhcnRpY2xlVHlwZSIsImFydGljbGVUeXBlIjoxfSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiWWFic3JhIE1lbGFrIiwiZmFtaWx5IjoiU2l0b3RlIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1pY2hhZWwgR2lyaW1heSIsImZhbWlseSI6IkdlYnJlbWVkaGluZSJ9fV0sImlkcyI6eyJpc3NuIjpbXSwiaXNibiI6W10sImRvaSI6IjEwLjIwOTQ0L3ByZXByaW50czIwMjQwOS4wNzA0LnYxIiwib3BlbmFsZXgiOiJXNDQwMjQ2NTQxNCIsImV4YSI6Imh0dHBzOi8vd3d3LnByZXByaW50cy5vcmcvbWFudXNjcmlwdC8yMDI0MDkuMDcwNCJ9LCJ0aXRsZSI6IkNvbXByZWhlbnNpdmUgUmV2aWV3IG9mIEV1dHJvcGhpY2F0aW9uIGluIEZyZXNod2F0ZXIgRWNvc3lzdGVtczogQ2F1c2VzLCBFZmZlY3RzLCBBc3Nlc3NtZW50LCBhbmQgTWFuYWdlbWVudCBTdHJhdGVnaWVzIiwiYWJzdHJhY3QiOiJUaGlzIHJldmlldyBleGFtaW5lcyBldXRyb3BoaWNhdGlvbiBpbiBmcmVzaHdhdGVyIGVjb3N5c3RlbXMsIGEgd2lkZXNwcmVhZCBlbnZpcm9ubWVudGFsIGlzc3VlIHN0ZW1taW5nIGZyb20gZXhjZXNzaXZlIG51dHJpZW50IGlucHV0LCBsYXJnZWx5IGR1ZSB0byBodW1hbiBhY3Rpdml0aWVzLiBUaGUgc3R1ZHkgZXhwbG9yZXMgdGhlIG9yaWdpbnMsIGNvbnNlcXVlbmNlcywgZXZhbHVhdGlvbiB0ZWNobmlxdWVzLCBhbmQgY29udHJvbCBtZXRob2RzIGZvciBldXRyb3BoaWNhdGlvbi4gUHJpbWFyeSBjYXVzZXMgaW5jbHVkZSBhZ3JpY3VsdHVyYWwgZGlzY2hhcmdlLCB1cmJhbiBkZXZlbG9wbWVudCwgYW5kIGNsaW1hdGUgc2hpZnRzLiBUaGUgZWZmZWN0cyBhcmUgZmFyLXJlYWNoaW5nLCBpbXBhY3RpbmcgYmlvZGl2ZXJzaXR5LCB3YXRlciBxdWFsaXR5LCBhbmQgc29jaW8tZWNvbm9taWMgZmFjdG9ycy4gVGhlIGFzc2Vzc21lbnQgdXRpbGl6ZXMgYSBjb21iaW5hdGlvbiBvZiBjaGVtaWNhbCwgcGh5c2ljYWwsIGFuZCBiaW9sb2dpY2FsIG1hcmtlcnMsIHdpdGggY3V0dGluZy1lZGdlIHRlY2hub2xvZ2llcyBsaWtlIHJlbW90ZSBzZW5zaW5nIGltcHJvdmluZyBtb25pdG9yaW5nIGVmZm9ydHMuIE1pdGlnYXRpb24gc3RyYXRlZ2llcyBhcmUgZGl2ZXJzZSwgaW5jb3Jwb3JhdGluZyBwcmV2ZW50YXRpdmUgYWN0aW9ucyAoc3VjaCBhcyBudXRyaWVudCBsaW1pdGF0aW9uIGFuZCBsYW5kIG1hbmFnZW1lbnQpLCBjb3JyZWN0aXZlIG1lYXN1cmVzIChpbmNsdWRpbmcgYmlvLW1hbmlwdWxhdGlvbiBhbmQgY2hlbWljYWwgYXBwbGljYXRpb25zKSwgYW5kIHJlZ3VsYXRvcnkgaW5pdGlhdGl2ZXMgKGxpa2Ugd2F0ZXIgcXVhbGl0eSBndWlkZWxpbmVzIGFuZCBudXRyaWVudCBleGNoYW5nZSBzY2hlbWVzKS4gV2hpbGUgY2FzZSBzdHVkaWVzIGRlbW9uc3RyYXRlIHN1Y2Nlc3NmdWwgaW50ZXJ2ZW50aW9ucywgdGhleSBhbHNvIHJldmVhbCBvbmdvaW5nIGNoYWxsZW5nZXMsIHBhcnRpY3VsYXJseSBlY29zeXN0ZW0gdW5wcmVkaWN0YWJpbGl0eSBhbmQgY2xpbWF0ZSBjaGFuZ2UgaW5mbHVlbmNlcy4gVGhlIHJlc2VhcmNoIGhpZ2hsaWdodHMgdGhlIHByb21pc2Ugb2YgZW1lcmdpbmcgdGVjaG5vbG9naWVzLCBub3RhYmx5IG1hY2hpbmUgbGVhcm5pbmcsIGluIGVuaGFuY2luZyBtYW5hZ2VtZW50IHByYWN0aWNlcy4gS25vd2xlZGdlIGdhcHMgcGVyc2lzdCwgZXNwZWNpYWxseSByZWdhcmRpbmcgZXV0cm9waGljYXRpb24tYmlvZGl2ZXJzaXR5IGR5bmFtaWNzIGluIHRyb3BpY2FsIGVudmlyb25tZW50cyBhbmQgdGhlIHNvY2lvLWVjb25vbWljIHJhbWlmaWNhdGlvbnMgb2YgaW50ZXJ2ZW50aW9ucy4gVGhlIHN0dWR5IGVtcGhhc2l6ZXMgdGhlIGltcG9ydGFuY2Ugb2YgZmxleGlibGUsIGNvbXByZWhlbnNpdmUgYXBwcm9hY2hlcyB0aGF0IGJsZW5kIHNjaWVudGlmaWMgaW5zaWdodHMgd2l0aCBwcmFjdGljYWwgc29sdXRpb25zIGFuZCBzdHJvbmcgcG9saWN5IHN0cnVjdHVyZXMgdG8gdGFja2xlIHRoaXMgaW50cmljYXRlIGVudmlyb25tZW50YWwgY2hhbGxlbmdlIGluIGFuIGVyYSBvZiBjbGltYXRlIGNoYW5nZS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjA5NDQvcHJlcHJpbnRzMjAyNDA5LjA3MDQudjEiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4yMDk0NC9wcmVwcmludHMyMDI0MDkuMDcwNC52MSIsImlzc3VlZERhdGUiOiIyMDI0LTA5LTEwIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMSIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiUHJlcHJpbnRzLm9yZyIsImltcGFjdEZhY3RvciI6IjEuMTUifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjAsImNpdGVkQnlDb3VudCI6NH19LCJxdW90ZSI6IkV1dHJvcGhpY2F0aW9uIGluIGZyZXNod2F0ZXIgc3lzdGVtcyBpcyBwcmltYXJpbHkgZHJpdmVuIGJ5IHRoZSBleGNlc3NpdmUgaW5wdXQgb2YgbnV0cmllbnRzLCBwYXJ0aWN1bGFybHkgbml0cm9nZW4gKE4pIGFuZCBwaG9zcGhvcnVzIChQKS4gVGhlc2UgbnV0cmllbnRzIG9mdGVuIGVudGVyIHdhdGVyIGJvZGllcyB0aHJvdWdoIGFncmljdWx0dXJhbCBydW5vZmYsIHdoZXJlIGZlcnRpbGl6ZXJzLCBsaXZlc3RvY2sgd2FzdGUsIGFuZCBzb2lsIGVyb3Npb24gYXJlIHNpZ25pZmljYW50IGNvbnRyaWJ1dG9ycyAoR3JpZmZpdGggJiBHb2JsZXIsIDIwMjApLiBVcmJhbiBhcmVhcyBhbHNvIGNvbnRyaWJ1dGUgdG8gbnV0cmllbnQgbG9hZGluZyB0aHJvdWdoIHVudHJlYXRlZCBvciBpbmFkZXF1YXRlbHkgdHJlYXRlZCB3YXN0ZXdhdGVyLCBzdG9ybXdhdGVyIHJ1bm9mZiwgYW5kIGluZHVzdHJpYWwgZWZmbHVlbnRzLCBhbGwgb2Ygd2hpY2ggY2Fycnkgc3Vic3RhbnRpYWwgYW1vdW50cyBvZiBudXRyaWVudHMgaW50byByaXZlcnMsIGxha2VzLCBhbmQgb3RoZXIgZnJlc2h3YXRlciBib2RpZXMgKFd1cnRzYmF1Z2ggZXQgYWwuLCAyMDE5KS4gTGFuZCB1c2UgY2hhbmdlcywgaW5jbHVkaW5nIHVyYmFuaXphdGlvbiwgZGVmb3Jlc3RhdGlvbiwgYW5kIGFncmljdWx0dXJhbCBleHBhbnNpb24sIGhhdmUgc2lnbmlmaWNhbnRseSBkaXNydXB0ZWQgbmF0dXJhbCBudXRyaWVudCBjeWNsZXMuIn1d. The pathway through which untreated urban sewage contributes to nutrient enrichment and bloom development is shown in Figure 4. Cities like Bengaluru, Hyderabad, and Delhi generate billions of liters daily, with lakes such as Bellandur and Hussainsagar acting as de facto sewage sinks, elevating total nitrogen to 10-20 mg/L and phosphorus to 1-5 mg/L, far exceeding eutrophic thresholds (Gobler et al., 2024; Nair & Nayak, 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkNocmlzdG9waGVyIEouIiwiZmFtaWx5IjoiR29ibGVyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlJ1dGggVy4iLCJmYW1pbHkiOiJEcmlua3dhdGVyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkEuIFcuIiwiZmFtaWx5IjoiQW50aG9ueSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJKZW5uaWZlciBBLiIsImZhbWlseSI6IkdvbGVza2kifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW5uIE1hcmllIEUuIiwiZmFtaWx5IjoiRmFtdWxhcm8tUGVjb3JhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1pY2hhZWwgQi4iLCJmYW1pbHkiOiJXYWxsYWNlIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5vcmEgUi4gVy4iLCJmYW1pbHkiOiJTdHJhcXVhZGluZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJSb25vam95IiwiZmFtaWx5IjoiSGVtIn19XSwiaWRzIjp7Imlzc24iOlsiMTY2NC0zMDJYIl0sImlzYm4iOltdLCJkb2kiOiIxMC4zMzg5L2ZtaWNiLjIwMjQuMTQ2NDY4NiIsInBtaWQiOiIzOTU2ODk5NyIsIm9wZW5hbGV4IjoiVzQ0MDQxMjA5NTIiLCJleGEiOiJodHRwczovL3d3dy5mcm9udGllcnNpbi5vcmcvam91cm5hbHMvbWljcm9iaW9sb2d5L2FydGljbGVzLzEwLjMzODkvZm1pY2IuMjAyNC4xNDY0Njg2L2Z1bGwifSwidGl0bGUiOiJTZXdhZ2UtYW5kIGZlcnRpbGl6ZXItZGVyaXZlZCBudXRyaWVudHMgYWx0ZXIgdGhlIGludGVuc2l0eSwgZGl2ZXJzaXR5LCBhbmQgdG94aWNpdHkgb2YgaGFybWZ1bCBjeWFub2JhY3RlcmlhbCBibG9vbXMgaW4gZXV0cm9waGljIGxha2VzIiwiYWJzdHJhY3QiOiJDeWFub2JhY3RlcmlhbCBoYXJtZnVsIGFsZ2FsIGJsb29tcyAoQ0hBQnMpIGFyZSBwcm9tb3RlZCBieSBleGNlc3NpdmUgbnV0cmllbnQgbG9hZGluZyBhbmQsIHdoaWxlIGZlcnRpbGl6ZXJzIGFuZCBzZXdhZ2UgYXJlIHRoZSBtb3N0IHByZXZhbGVudCBleHRlcm5hbCBudXRyaWVudCBzb3VyY2VzIGluIG1vc3Qgd2F0ZXJzaGVkcywgdGhlIGRpZmZlcmVudGlhbCBlZmZlY3RzIG9mIHRoZXNlIG51dHJpZW50IHNvdXJjZXMgb24gQ0hBQnMgYXJlIHVua25vd24uIEhlcmUsIHdlIHRyYWNrZWQgQ0hBQnMgYW5kIHBlcmZvcm1lZCBleHBlcmltZW50cyBpbiBmaXZlIGRpc3RpbmN0IGxha2VzIGFjcm9zcyB0aGUgTm9ydGhlcm4gVVMgaW5jbHVkaW5nIExha2UgRXJpZS4gRmVydGlsaXplcnMgd2l0aCBhbW1vbml1bSBhbmQgb3J0aG9waG9zcGhhdGUsIG1lbWJyYW5lICgwLjIgzrxtKS1maWx0ZXJlZCBzZXdhZ2UgKGRvbWluYXRlZCBieSByZWR1Y2VkIGZvcm1zIG9mIG5pdHJvZ2VuKSBzYW5kLWFuZCBtZW1icmFuZS1maWx0ZXJlZCBzZXdhZ2UgKGRvbWluYXRlZCBieSBuaXRyYXRlKSwgYW5kIGFuIGlub3JnYW5pYyBudXRyaWVudCBzb2x1dGlvbiBvZiBhbW1vbml1bSBhbmQgb3J0aG9waG9zcGhhdGUgd2VyZSB1c2VkIGFzIGV4cGVyaW1lbnRhbCBudXRyaWVudCBzb3VyY2VzIGZvciBDSEFCcyBhdCBOLWVxdWl2YWxlbnQsIGVudmlyb25tZW50YWxseSByZWFsaXN0aWMgY29uY2VudHJhdGlvbnMuIFBoeXRvcGxhbmt0b24gY29tbXVuaXRpZXMgd2VyZSBldmFsdWF0ZWQgZmx1b3JvbWV0cmljYWxseSwgbWljcm9zY29waWNhbGx5LCBhbmQgdmlhIGhpZ2ggdGhyb3VnaHB1dCBzZXF1ZW5jaW5nIG9mIHRoZSAxNlMgclJOQSBnZW5lLCBhbmQgbGV2ZWxzIG9mIG1pY3JvY3lzdGluIGFuZCB0aGUgzrQgMTUgTiBjb250ZW50IG9mIHBhcnRpY3VsYXRlIG9yZ2FuaWMgbml0cm9nZW4gKM60UE8gMTUgTikgd2VyZSBxdWFudGlmaWVkLiBGZXJ0aWxpemVyIGFuZCBib3RoIHNvdXJjZXMgb2Ygd2FzdGV3YXRlciBpbmNyZWFzZWQgdGhlIGFidW5kYW5jZSBvZiBjeWFub2JhY3RlcmlhIGluIGFsbCBleHBlcmltZW50cyBhY3Jvc3MgYWxsIGZpdmUgbGFrZXMgKCBwICZhbXA7bHQ7IDAuMDUgZm9yIGFsbCkgd2hlcmVhcyBlZmZlY3RzIG9uIGV1a2FyeW90aWMgcGh5dG9wbGFua3RvbiB3ZXJlIGxpbWl0ZWQuIFNhbmQtZmlsdGVyZWQgc2V3YWdlIGNvbnRhaW5lZCBsZXNzIFAsIG9yZ2FuaWMgbWF0dGVyLCBhbmQgYW1tb25pdW0gYnV0IG1vcmUgbml0cmF0ZSBhbmQgaGFkIGEgMjUlIGxlc3MgcG90ZW50IHN0aW11bGF0b3J5IGVmZmVjdCBvbiBjeWFub2JhY3RlcmlhIHRoYW4gbWVtYnJhbmUtZmlsdGVyZWQgc2V3YWdlLCBzdWdnZXN0aW5nIG5pdHJpZmljYXRpb24gbWF5IHBsYXkgYSByb2xlIGluIHJlZHVjaW5nIENIQUJzLiBGZXJ0aWxpemVyIGluY3JlYXNlZCBtaWNyb2N5c3RpbiBsZXZlbHMgYW5kIGRlY3JlYXNlZCB0aGUgzrRQTyAxNSBOIHdoZXJlYXMgd2FzdGV3YXRlciBpbmNyZWFzZWQgzrRQTyAxNSBOICggcCAmYW1wO2x0OyAwLjA1IGZvciBhbGwpLiBNaWNyb2N5c3RpcyB3YXMgdGhlIGdlbnVzIG1vc3QgY29uc2lzdGVudGx5IHByb21vdGVkIGJ5IG51dHJpZW50IHNvdXJjZXMgKCBwICZhbXA7bHQ7IDAuMDUgaW4gYWxsIGV4cGVyaW1lbnRzKSwgZm9sbG93ZWQgYnkgQ3lhbm9iaXVtICggcCAmYW1wO2x0OyAwLjA1IGluIDUwJSBvZiBleHBlcmltZW50cyksIHdpdGggaW5jcmVhc2VzIGluIE1pY3JvY3lzdGlzIGJpb21hc3MgY29uc2lzdGVudGx5IGVsaWNpdGVkIGJ5IG1lbWJyYW5lLWZpbHRlcmVkIHdhc3Rld2F0ZXIuIENvbGxlY3RpdmVseSwgcmVzdWx0cyBkZW1vbnN0cmF0ZSB0aGF0IGRpZmZlcmluZyB0eXBlcyBvZiBzZXdhZ2UgZGlzY2hhcmdlIGFuZCBmZXJ0aWxpemVycyBjYW4gcHJvbW90ZSBDSEFCIGludGVuc2l0eSBhbmQgdG94aWNpdHksIHdoaWxlIGNvbmN1cnJlbnRseSBhbHRlcmluZyBDSEFCIGRpdmVyc2l0eSBhbmQgzrRQTyAxNSBOLiBXaGlsZSBtZW1icmFuZS1maWx0ZXJlZCBzZXdhZ2UgY29uc2lzdGVudGx5IGZhdm9yZWQgTWljcm9jeXN0aXMgLCB0aGUgZGlzY2hhcmdlIG9mIHNld2FnZSB0aHJvdWdoIHNhbmRzIG11dGVkIGJsb29tIGludGVuc2l0eSBzdWdnZXN0aW5nIHNhbmQtYmVkcyBtYXkgcmVwcmVzZW50IGEgdG9vbCB0byByZW1vdmUga2V5IG51dHJpZW50cyBhbmQgcGFydGlhbGx5IG1pdGlnYXRlIENIQUJzLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzg5L2ZtaWNiLjIwMjQuMTQ2NDY4NiIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cuZnJvbnRpZXJzaW4ub3JnL2pvdXJuYWxzL21pY3JvYmlvbG9neS9hcnRpY2xlcy8xMC4zMzg5L2ZtaWNiLjIwMjQuMTQ2NDY4Ni9wZGYiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMS0wNiIsImFjY2Vzc2VkRGF0ZSI6IjIwMjYtMDIiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZyb250aWVycyBpbiBNaWNyb2Jpb2xvZ3kiLCJ2b2x1bWUiOjE1LCJpbXBhY3RGYWN0b3IiOiIzLjk5In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTQ2NDY4NiwibGFzdFBhZ2UiOjE0NjQ2ODZ9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRnJvbnRpZXJzIE1lZGlhIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo3MywiY2l0ZWRCeUNvdW50Ijo5fX0sInF1b3RlIjoiZGVyaXZlZCBudXRyaWVudHMgYWx0ZXIgdGhlIGludGVuc2l0eSwgZGl2ZXJzaXR5LCBhbmQgdG94aWNpdHkgb2YgaGFybWZ1bCBjeWFub2JhY3RlcmlhbCBibG9vbXMgLi4uIEN5YW5vYmFjdGVyaWFsIGhhcm1mdWwgYWxnYWwgYmxvb21zIChDSEFCcykgYXJlIHByb21vdGVkIGJ5IGV4Y2Vzc2l2ZSBudXRyaWVudCBsb2FkaW5nIGFuZCwgd2hpbGUgZmVydGlsaXplcnMgYW5kIHNld2FnZSBhcmUgdGhlIG1vc3QgcHJldmFsZW50IGV4dGVybmFsIG51dHJpZW50IHNvdXJjZXMgaW4gbW9zdCB3YXRlcnNoZWRzLCB0aGUgZGlmZmVyZW50aWFsIGVmZmVjdHMgb2YgdGhlc2UgbnV0cmllbnQgLi4uIG9yZ2FuaWMgbml0cm9nZW4gKM60UE8xNU4pIHdlcmUgcXVhbnRpZmllZC4gRmVydGlsaXplciBhbmQgYm90aCBzb3VyY2VzIG9mIHdhc3Rld2F0ZXIgaW5jcmVhc2VkIHRoZSBhYnVuZGFuY2Ugb2YgLi4uIHBoeXRvcGxhbmt0b24gd2VyZSBsaW1pdGVkLiBTYW5kLWZpbHRlcmVkIHNld2FnZSBjb250YWluZWQgbGVzcyBQLCBvcmdhbmljIG1hdHRlciwgYW5kIGFtbW9uaXVtIGJ1dCBtb3JlIG5pdHJhdGUgYW5kIC4uLiBkaWZmZXJpbmcgdHlwZXMgb2Ygc2V3YWdlIGRpc2NoYXJnZSBhbmQgZmVydGlsaXplcnMgY2FuIHByb21vdGUgQ0hBQiBpbnRlbnNpdHkgYW5kIHRveGljaXR5LCB3aGlsZSBjb25jdXJyZW50bHkgLi4uIHNld2FnZSB0aHJvdWdoIHNhbmRzIG11dGVkIGJsb29tIGludGVuc2l0eSBzdWdnZXN0aW5nIHNhbmQtYmVkcyBtYXkgcmVwcmVzZW50IGEgdG9vbCB0byByZW1vdmUga2V5IG51dHJpZW50cyBhbmQgLi4uICggV2F0c29uIGV0IGFsLiwgMjAxNiA7IEJ1bGxlcmphaG4gZXQgYWwuLCAyMDE2IDsgSHVpc21hbiBldCBhbC4sIDIwMTggKSBhbmQgZXhjZXNzaXZlIG51dHJpZW50IGxvYWRpbmcgaXMgdGhlIHByb2Nlc3MgbW9zdCBjb21tb25seSBjaXRlZCBhcyB0aGUgZW52aXJvbm1lbnRhbCBmYWN0b3IgcHJvbW90aW5nIENIQUJzICgifSx7Im9yaWdpbiI6NCwid29yayI6eyJ0eXBlIjo4LCJzdWJ0eXBlIjp7Im9uZW9mS2luZCI6ImFydGljbGVUeXBlIiwiYXJ0aWNsZVR5cGUiOjF9LCJjb250cmlidXRvcnMiOlt7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJOYXZ5YSBWaWtyYW1hbiIsImZhbWlseSI6Ik5haXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUHJhdGVlcCBLdW1hciIsImZhbWlseSI6Ik5heWFrIn19XSwiaWRzIjp7Imlzc24iOlsiMjI5Ni03NzQ1Il0sImlzYm4iOltdLCJkb2kiOiIxMC4zMzg5L2ZtYXJzLjIwMjMuMTA4NzI5NiIsIm9wZW5hbGV4IjoiVzQzODI3MjMxMzMifSwidGl0bGUiOiJVbmNvdmVyaW5nIHdhdGVyIHF1YWxpdHkgYW5kIGV2YWx1YXRpbmcgdnVsbmVyYWJpbGl0aWVzIG9mIHNtYWxsLXNjYWxlIGZpc2hlcmllcyBpbiBDaGlsaWthIExhZ29vbiwgSW5kaWEiLCJhYnN0cmFjdCI6IlNtYWxsLXNjYWxlIGZpc2hlcmllcyAoU1NGcykgcGxheSBhIHN0cm9uZyByb2xlIGluIHN1c3RhaW5pbmcgbWlsbGlvbnMgb2YgbGl2ZWxpaG9vZHMsIGZvb2Qgc2VjdXJpdHksIG51dHJpdGlvbiwgYW5kIGluY29tZSBnbG9iYWxseSBidXQgdGhlIGZpc2hlcnMgZW5nYWdlZCBpbiB0aGlzIHNlY3RvciBzaW11bHRhbmVvdXNseSBleHBlcmllbmNlIGhpZ2ggbGV2ZWxzIG9mIHZ1bG5lcmFiaWxpdHkgYW5kIHByb2Nlc3NlcyBvZiBtYXJnaW5hbGlzYXRpb24uIFNldmVyYWwgZmFjdG9ycyBhcmUgYXR0cmlidXRlZCB0byB0aGUgbXVsdGlkaW1lbnNpb25hbCB2dWxuZXJhYmlsaXRpZXMgdGhlIHNtYWxsLXNjYWxlIGZpc2hlcnMgZXhwZXJpZW5jZSwgc3Bhbm5pbmcgYm90aCBuYXR1cmFsIChlLmcuLCBuYXR1cmFsIGRpc2FzdGVycywgZWNvc3lzdGVtIGNoYW5nZSkgYW5kIGFudGhyb3BvZ2VuaWMgKGUuZy4sIHBvbGljeSBjaGFuZ2UsIGh5ZHJvbG9naWNhbCBpbnRlcnZlbnRpb25zLCBhcXVhY3VsdHVyZSkgcHJlc3N1cmVzLiBXaGlsZSB0aGVyZSBpcyBtdWNoIGxpdGVyYXR1cmUgb24gdmFyaW91cyBuYXR1cmFsIGFuZCBodW1hbiBkcml2ZXJzIG9mIHZ1bG5lcmFiaWxpdHkgaW4gc21hbGwtc2NhbGUgZmlzaGVyeSBjb21tdW5pdGllcywgYW4gYWJzZW5jZSBvZiByZXNlYXJjaCBjb25uZWN0aW5nIHZ1bG5lcmFiaWxpdHkgd2l0aCB3YXRlciBxdWFsaXR5IGlzIGV2aWRlbnQuIEZpc2hlciBjb21tdW5pdGllcyBvZnRlbiB0YWxrIGFib3V0IGZpc2ggaW4gcmVsYXRpb24gdG8gdGhlIGhlYWx0aCBvZiB0aGVpciBhcXVhdGljIGhhYml0YXRzIHdoZXJlaW4gd2F0ZXIgcXVhbGl0eSBpcyBzZWVuIGFzIGEga2V5IHBhcmFtZXRlci4gVGhlIGxpbmsgYmV0d2VlbiBoZWFsdGh5IGZpc2ggYW5kIGdvb2QgcXVhbGl0eSB3YXRlciBoYXMgc2lnbmlmaWNhbnQgaW1wbGljYXRpb25zIGZvciBzdHJvbmcgYW5kIHZpYWJsZSBmaXNoaW5nIGNvbW11bml0aWVzLiBUaGlzIHBhcGVyIGV4YW1pbmVzIHRoZXNlIGxpbmtzIGZ1cnRoZXIgYnkgZm9jdXNpbmcgb24gdGhlIG5hdHVyZSBvZiB2dWxuZXJhYmlsaXRpZXMgY2F1c2VkIGJ5IHdhdGVyIHF1YWxpdHkgY2hhbmdlcyBpbiB0aGUgc21hbGwtc2NhbGUgZmlzaGVyeSBzeXN0ZW0gb2YgQ2hpbGlrYSBMYWdvb24gaW4gSW5kaWEuIFdlIHVuZGVydGFrZSBkZXRhaWxlZCBhbmFseXNpcyBvZiB0aGUgaW52YXNpdmUgc2hyaW1wIGFxdWFjdWx0dXJlIGFjdGl2aXRpZXMgYW5kIGh5ZHJvbG9naWNhbCBpbnRlcnZlbnRpb25zIGZvciBvcGVuaW5nIG9mIGEgbGFnb29uIGlubGV0IHdpdGggdGhlIEJheSBvZiBCZW5nYWwgYXMgdHdvIGRvbWluYW50IGRyaXZlcnMgYWR2ZXJzZWx5IGltcGFjdGluZyB3YXRlciBxdWFsaXR5IGFuZCBpbmNyZWFzaW5nIHZ1bG5lcmFiaWxpdGllcyBvZiB0aGUgZW50aXJlIHNtYWxsLXNjYWxlIGZpc2hlcmllcyBzb2NpYWwtZWNvbG9naWNhbCBzeXN0ZW0uIE91ciBhbmFseXNpcyBzdWdnZXN0cyB0aGF0IHRoZXJlIGFyZSBzdHJvbmcgaW50ZXJjb25uZWN0aW9ucyBiZXR3ZWVuIGNoYW5nZXMgaW4gd2F0ZXIgcXVhbGl0eSBhbmQgdGhlIGxldmVscyBvZiB2dWxuZXJhYmlsaXRpZXMgaW4gdGhlIFNTRnMgb2YgQ2hpbGlrYSBMYWdvb24uIFBvbGx1dGFudHMgc3VjaCBhcyBwZXN0aWNpZGVzLCBhbmQgb3JnYW5pYyBjb21wb3VuZHMgYWNjdW11bGF0ZSBpbiBmaXNoIHRpc3N1ZXMgYW5kIGFmZmVjdCB0aGVpciBncm93dGgsIHJlcHJvZHVjdGlvbiwgYW5kIG92ZXJhbGwgaGVhbHRoLiBUaGlzIGxlZCB0byBkZWNsaW5lcyBpbiBmaXNoIHBvcHVsYXRpb25zLCBtYWtpbmcgaXQgbW9yZSBkaWZmaWN1bHQgZm9yIGZpc2hlcnMgdG8gbWFrZSBhIGxpdmluZy4gSW4gYWRkaXRpb24gdG8gZGlyZWN0IGltcGFjdHMgb24gZmlzaCBwb3B1bGF0aW9ucywgcG9vciB3YXRlciBxdWFsaXR5IGFsc28gaGFzIGluZGlyZWN0IGVmZmVjdHMgb24gdGhlIHNvY2lhbCBhbmQgZWNvbm9taWMgdnVsbmVyYWJpbGl0eSBvZiBTU0ZzLiBGb3IgZXhhbXBsZSwgY29udGFtaW5hdGlvbiBvZiB3YXRlciBzb3VyY2VzIGxlZCB0byB0aGUgcmVkdWNlZCBudW1iZXIgb2YgZmlzaCBzcGVjaWVzIHJlZHVjaW5nIHRoZSBhbW91bnQgb2YgdGltZSBmaXNoZXJzIGNhbiBzcGVuZCBvbiBmaXNoaW5nIGFjdGl2aXRpZXMuIFRoaXMgYWxzbyBhZmZlY3RlZCB0aGUgbWFya2V0YWJpbGl0eSBvZiBmaXNoIHByb2R1Y3RzLCByZWR1Y2VkIGluY29tZSBhbmQgaW5jcmVhc2VkIHBvdmVydHkuIFRvIGZ1bGx5IHVuZGVyc3RhbmQgdGhlIGludGVyY29ubmVjdGlvbnMgYmV0d2VlbiB3YXRlciBxdWFsaXR5IGFuZCB2dWxuZXJhYmlsaXR5IGluIFNTRnMgaW4gQ2hpbGlrYSBMYWdvb24sIGl0IGlzIGltcG9ydGFudCB0byBjb25zaWRlciBib3RoIGVudmlyb25tZW50YWwgYW5kIHNvY2lhbCBmYWN0b3JzLCBhcyB3ZWxsIGFzIHRoZSBjb21wbGV4IGZlZWRiYWNrIGxvb3BzIGJldHdlZW4gdGhlc2UgZmFjdG9ycy4gVGhlIHN0dWR5IGhlbHBzIGluIGJyaWRnaW5nIGEgY3J1Y2lhbCBnYXAgaW4gb3VyIHVuZGVyc3RhbmRpbmcgb2YgdGhlIHJvbGUgb2Ygd2F0ZXIgcXVhbGl0eSBpbiB2dWxuZXJhYmlsaXR5IGFuYWx5c2lzIHdpdGhpbiByZXNvdXJjZSBkZXBlbmRlbnQgY29tbXVuaXRpZXMuIFdlIGNvbmNsdWRlIHdpdGgga2V5IGluc2lnaHRzIG9uIHBvc3NpYmxlIGNvcGluZyByZXNwb25zZXMgYW5kIGFkYXB0aXZlIGNhcGFjaXR5IG5lY2Vzc2FyeSBmb3IgdGhlIHNtYWxsLXNjYWxlIGZpc2hlcmllcyBjb21tdW5pdGllcyB0byB0cmFuc2l0aW9uIHRvd2FyZCB2aWFiaWxpdHkuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjMzODkvZm1hcnMuMjAyMy4xMDg3Mjk2IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5mcm9udGllcnNpbi5vcmcvYXJ0aWNsZXMvMTAuMzM4OS9mbWFycy4yMDIzLjEwODcyOTYvcGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjMtMDYtMjkiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTEwIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJGcm9udGllcnMgaW4gTWFyaW5lIFNjaWVuY2UiLCJ2b2x1bWUiOjEwLCJpbXBhY3RGYWN0b3IiOiI0LjI4In0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkZyb250aWVycyBNZWRpYSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NTEsImNpdGVkQnlDb3VudCI6NX19LCJxdW90ZSI6IldhdGVyIHF1YWxpdHkgb2YgQ2hpbGlrYSBMYWdvb24gaGFzIGRlZ3JhZGVkIGR1ZSB0byBhbnRocm9wb2dlbmljIGludGVydmVudGlvbnMgYW5kIG5hdHVyYWwgZm9yY2VzIG9mIGNoYW5nZS4gVmFyaW91cyBhY3Rpdml0aWVzIGluIHRoZSBDaGlsaWthIExhZ29vbiwgc3VjaCBhcyBpbmR1c3RyaWFsIHdhc3Rld2F0ZXIgZGlzcG9zYWwsIHNld2FnZSBkdW1waW5nLCBhcXVhY3VsdHVyZSwgaHlkcm9sb2dpY2FsIGludGVydmVudGlvbnMsIGFuZCBjeWNsb25lcywgYXJlIHByb2R1Y2luZyBzYWxpbml0eSBjaGFuZ2VzLCBzZWRpbWVudCBkZXBvc2l0aW9uLCBudXRyaWVudCBlbnJpY2htZW50LCBldXRyb3BoaWNhdGlvbiwgYW5kIGRlYWQgem9uZXMsIGFzIHN0YXRlZC4gRmlzaCBzdXJ2aXZhbCBpcyBhdCByaXNrIGFzIGEgcmVzdWx0IG9mIHRoZSBzaWduaWZpY2FudCByZWR1Y3Rpb24gaW4gZW52aXJvbm1lbnRhbCBjaXJjdW1zdGFuY2VzLCB3aGljaCBkaW1pbmlzaGVzIGZpc2hlcm1lbuKAmXMgaW5jb21lIGFuZCBwdXNoZXMgdGhlbSBvdXQgb2Ygc29jaWV0eS4gVGhlc2UgY2F1c2VzIGFmZmVjdGluZyB0aGUgd2F0ZXIgcXVhbGl0eSBvZiB0aGUgTGFnb29uIGRlcGljdCBtYW55IGZhY2VzIG9mIHZ1bG5lcmFiaWxpdHkgaW4gU1NGIGNvbW11bml0aWVzLiBBcyB0aGUgQ2hpbGlrYSBMYWdvb24gZW52aXJvbm1lbnQgaXMgZXhwZWN0ZWQgdG8gdW5kZXJnbyBpbmNyZWFzaW5nbHkgZHJhc3RpYyBjaGFuZ2VzIChzdWNoIGFzIGNhdGNoIGNvbXBvc2l0aW9uLCBjYXRjaCBjYXBhY2l0eSwgYmlvZGl2ZXJzaXR5IHZhcmlhdGlvbnMsIHdhdGVyIHF1YWxpdHksIGFuZCBmaXNoZXJ5IHJldmVudWVzKSwgaW50ZXJuYXRpb25hbCBwb2xpY3kgaGFzIGJlY29tZSBtb3JlIGludGVyZXN0ZWQgaW4gYWRhcHRhdGlvbiBtZWFzdXJlcy4gR2l2ZW4gaG93IGRpdmVyc2UgU1NGIGVjb3N5c3RlbXMgYXJlIGV4cGVjdGVkIHRvIGJlIGJldHdlZW4gY29udGluZW50cywgdGhlcmUgaXMgbm8g4oCcb25lIHNpemUgZml0cyBhbGzigJ0gYXBwcm9hY2guIFRoZSBwYXBlciBvdXRsaW5lZCBzZXZlcmFsIGNvcGluZyBhbmQgYWRhcHRhdGlvbiBtZWNoYW5pc21zIHRvIGltcHJvdmUgd2F0ZXIgcXVhbGl0eSBhbmQgY29udGludWUgZmlzaGluZyBpbiBhIHN1c3RhaW5hYmxlIHdheS4gQWRhcHRhdGlvbiBzdHJhdGVnaWVzIGNhbiBvY2Nhc2lvbmFsbHkgaGF2ZSB1bmZvcmVzZWVuIGNvbnNlcXVlbmNlcyBmb3IgZmlzaGVyaWVzIGFuZCBmaXNoaW5nIGNvbW11bml0aWVzLiBBbnkgdW5leHBlY3RlZCBjb25zZXF1ZW5jZXMgb2YgYWRhcHRhdGlvbiBjYW4gYmUgbWl0aWdhdGVkIHRocm91Z2ggbG9uZy10ZXJtIHBsYW5uaW5nIGFuZCBpZGVudGlmaWNhdGlvbiBvZiBmdXR1cmUgb3B0aW9ucyB0aHJvdWdoIHNjZW5hcmlvIGFuYWx5c2lzLiBUaGUgZmluZGluZ3Mgb2YgdGhpcyBwYXBlciBjYW4gYmUgdXRpbGl6ZWQgdG8gZGV2ZWxvcCBwb2xpY2llcyB0byBwcm90ZWN0IHdhdGVyIHF1YWxpdHkgYW5kIFNTRnMgaW4gYSB2YXJpZXR5IG9mIGNvYXN0YWwgbGFnb29uIHNldHRpbmdzLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyMi4yODUwMzYwODcwMzYxMzMsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40OTMwNTM0MzYyNzkzLCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyMy43NzM1MTk1MTU5OTEyMSwibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ4MzM2NDEwNTIyNDYxLCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyNS4yNTU1ODQ3MTY3OTY4NzUsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40ODk1NTkxNzM1ODM5ODQsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjI2Ljc0NDA2ODE0NTc1MTk1MywibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjUwNjgwNTQxOTkyMTg3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjguMjI2MTMzMzQ2NTU3NjE3LCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDkwNDYzMjU2ODM1OTQsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjI5LjcxNDYxNjc3NTUxMjY5NSwibGVmdCI6OC4zMzE1OTI1NTk4MTQ0NTMsIndpZHRoIjo0MC40OTQwOTg2NjMzMzAwOCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MzEuMTk2NjgzODgzNjY2OTkyLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDk0MzkyMzk1MDE5NTMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjMyLjY4NTE2NTQwNTI3MzQ0LCJsZWZ0Ijo4LjMzMTU5NjM3NDUxMTcxOSwid2lkdGgiOjQwLjQ4NzQ0NTgzMTI5ODgzLCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozNC4xNjcyMzI1MTM0Mjc3MzQsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDk3NzQ1NTEzOTE2MDE2LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozNS42NTU3MTU5NDIzODI4MSwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40OTg2MzgxNTMwNzYxNywiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MzcuMTM3NzgzMDUwNTM3MTEsImxlZnQiOjEwLjY4NTE4NjM4NjEwODM5OCwid2lkdGgiOjM4LjE0MTEwNTY1MTg1NTQ3LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozOC42MjYyNjY0Nzk0OTIxOSwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40ODcxNTIwOTk2MDkzNzUsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQwLjEwODMzMzU4NzY0NjQ4NCwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40ODk4NTY3MTk5NzA3LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo0MS41OTY4MTMyMDE5MDQzLCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4ODY2NjUzNDQyMzgzLCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo0My4wNzg4ODAzMTAwNTg1OTQsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDgxODYxMTE0NTAxOTUsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQ0LjU2NzM2MzczOTAxMzY3LCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4MzA1ODkyOTQ0MzM2LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo0Ni4wNDk0MzA4NDcxNjc5NywibGVmdCI6OC4zMzMxMDg5MDE5Nzc1MzksIndpZHRoIjo0MC41MDA3NDc2ODA2NjQwNiwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDcuNTM3OTE0Mjc2MTIzMDUsImxlZnQiOjguMzMzMTA4OTAxOTc3NTM5LCJ3aWR0aCI6NDAuNDk1NjA1NDY4NzUsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQ5LjAxOTk4MTM4NDI3NzM0NCwibGVmdCI6OC4zMzMxMDYwNDA5NTQ1OSwid2lkdGgiOjQwLjUwNjgxNjg2NDAxMzY3LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo1MC41MDg0NjQ4MTMyMzI0MiwibGVmdCI6OC4zMzMxMDYwNDA5NTQ1OSwid2lkdGgiOjQwLjQ4ODUwNjMxNzEzODY3LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo1MS45OTA1MjgxMDY2ODk0NSwibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ5NzcyNjQ0MDQyOTY5LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo1My40NzkwMTE1MzU2NDQ1MywibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjMzLjE1OTQ0NjcxNjMwODU5NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NTQuOTYxMDc4NjQzNzk4ODMsImxlZnQiOjEwLjY4MzY3MDA0Mzk0NTMxMiwid2lkdGgiOjM4LjEzNjU5NjY3OTY4NzUsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjU2LjQ0OTU2MjA3Mjc1MzkwNiwibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ4NzE0NDQ3MDIxNDg0NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIxMyJ9fV0%3D. This continuous influx disrupts stoichiometric balances, often favoring phosphorus excess relative to nitrogen, which paradoxically promotes nitrogen-fixing cyanobacteria like Dolichospermum and Aphanizomenon through atmospheric N₂ fixation (Haq et al., 2024; Nõges et al., 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjQsInN1YnR5cGUiOnt9LCJjb250cmlidXRvcnMiOlt7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJadWxmcWFydWwiLCJmYW1pbHkiOiJIYXEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU25vYmVyIiwiZmFtaWx5IjoiSXJzaGFkIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkF6bWF0IEFsYW0iLCJmYW1pbHkiOiJLaGFuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNoZWlraCBGLiIsImZhbWlseSI6IkFobWFkIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNob3drZWVuIE11emFtaWwiLCJmYW1pbHkiOiJCYXNoaXIifX1dLCJpZHMiOnsiaXNzbiI6WyIyNjMyLTA1MTciXSwiaXNibiI6W10sImRvaSI6IjEwLjU3NzIvaW50ZWNob3Blbi4xMDA2NTc4Iiwib3BlbmFsZXgiOiJXNDQwNDUyOTIwMyJ9LCJ0aXRsZSI6IkFkdmFuY2VzIGluIE1hbmFnaW5nIE5pdHJvZ2VuIGFuZCBQaG9zcGhvcnVzIEVtaXNzaW9ucyBpbiBSdW1pbmFudHM6IEEgSG9saXN0aWMgQXBwcm9hY2giLCJhYnN0cmFjdCI6IuKAnEJhbGFuY2luZyBOdXRyaXRpb24gYW5kIEVudmlyb25tZW50YWwgU3VzdGFpbmFiaWxpdHk6IEFkdmFuY2VzIGluIE1hbmFnaW5nIE5pdHJvZ2VuIGFuZCBQaG9zcGhvcnVzIEVtaXNzaW9ucyBpbiBSdW1pbmFudHPigJ0gZXhwbG9yZXMgdGhlIGludHJpY2F0ZSByZWxhdGlvbnNoaXAgYmV0d2VlbiBydW1pbmFudCBudXRyaXRpb24gYW5kIGVudmlyb25tZW50YWwgc3Rld2FyZHNoaXAuIEl0IGRlbHZlcyBpbnRvIHRoZSBkeW5hbWljcyBvZiBuaXRyb2dlbiBhbmQgcGhvc3Bob3J1cyBtZXRhYm9saXNtIGluIHJ1bWluYW50cywgYWRkcmVzc2luZyB0aGUgZW52aXJvbm1lbnRhbCByZXBlcmN1c3Npb25zIG9mIHRoZWlyIGVtaXNzaW9ucy4gVGhlc2UgZW1pc3Npb25zLCBwcmltYXJpbHkgaW4gdGhlIGZvcm0gb2YgYW1tb25pYSBhbmQgcGhvc3BoYXRlcywgcG9zZSBzaWduaWZpY2FudCBlbnZpcm9ubWVudGFsIGNoYWxsZW5nZXMsIGNvbnRyaWJ1dGluZyB0byBhaXIgYW5kIHdhdGVyIHBvbGx1dGlvbiwgc29pbCBkZWdyYWRhdGlvbiwgYW5kIGNsaW1hdGUgY2hhbmdlLiBUaGUgY2hhcHRlciBoaWdobGlnaHRzIGlubm92YXRpdmUgc3RyYXRlZ2llcyBhbmQgdGVjaG5vbG9naWNhbCBicmVha3Rocm91Z2hzIGFpbWVkIGF0IG1pdGlnYXRpbmcgbml0cm9nZW4gYW5kIHBob3NwaG9ydXMgZXhjcmV0aW9uLCBjcnVjaWFsIGZvciByZWR1Y2luZyBlbnZpcm9ubWVudGFsIHBvbGx1dGlvbi4gSXQgb2ZmZXJzIGluc2lnaHRzIGludG8gZGlldGFyeSBhZGp1c3RtZW50cywgZmVlZCBhZGRpdGl2ZXMsIGFuZCBiZXN0IHByYWN0aWNlcyBmb3Igc3VzdGFpbmFibGUgcnVtaW5hbnQgcHJvZHVjdGlvbi4gS2V5IGFyZWFzIG9mIGZvY3VzIGluY2x1ZGUgZGlldGFyeSBhZGp1c3RtZW50cyB0YWlsb3JlZCB0byBvcHRpbWl6ZSBudXRyaWVudCB1dGlsaXphdGlvbiBhbmQgcmVkdWNlIHdhc3RlLCB0aGUgdXRpbGl6YXRpb24gb2YgZmVlZCBhZGRpdGl2ZXMgdG8gZW5oYW5jZSBudXRyaWVudCBhYnNvcnB0aW9uIGFuZCBtaW5pbWl6ZSBleGNyZXRpb24sIGFuZCB0aGUgaW1wbGVtZW50YXRpb24gb2YgYmVzdCBtYW5hZ2VtZW50IHByYWN0aWNlcyB0byBwcm9tb3RlIHN1c3RhaW5hYmxlIHJ1bWluYW50IHByb2R1Y3Rpb24uIEJ5IGludGVncmF0aW5nIHNjaWVudGlmaWMgZmluZGluZ3Mgd2l0aCBwcmFjdGljYWwgYXBwbGljYXRpb25zLCB0aGlzIHJldmlldyB1bmRlcnNjb3JlcyB0aGUgaW1wb3J0YW5jZSBvZiBhY2hpZXZpbmcgaGFybW9ueSBiZXR3ZWVuIG51dHJpdGlvbiBvcHRpbWl6YXRpb24gYW5kIGVudmlyb25tZW50YWwgcmVzcG9uc2liaWxpdHkgaW4gbW9kZXJuIGFncmljdWx0dXJlLiBUaHJvdWdoIHRoaXMgY29tcHJlaGVuc2l2ZSBhbmFseXNpcywgdGhlIGNoYXB0ZXIgYWltcyBmb3IgYmFsYW5jaW5nIHRoZSBkdWFsIGdvYWxzIG9mIG9wdGltaXppbmcgcnVtaW5hbnQgbnV0cml0aW9uIGFuZCBtaW5pbWl6aW5nIGVudmlyb25tZW50YWwgaW1wYWN0LCB0aGVyZWJ5IGNvbnRyaWJ1dGluZyB0byBtb3JlIHN1c3RhaW5hYmxlIGFuZCByZXNpbGllbnQgYWdyaWN1bHR1cmFsIHN5c3RlbXMuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjU3NzIvaW50ZWNob3Blbi4xMDA2NTc4IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5pbnRlY2hvcGVuLmNvbS9jaXRhdGlvbi1wZGYtdXJsLzExODc4MTQiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMS0xNSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IlZldGVyaW5hcnkgbWVkaWNpbmUgYW5kIHNjaWVuY2UiLCJpbXBhY3RGYWN0b3IiOiIwLjk5In0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkludGVjaE9wZW4ifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjUyLCJjaXRlZEJ5Q291bnQiOjF9fSwicXVvdGUiOiJXYXRlciBRdWFsaXR5IENoYW5nZXMg4oCiIENsYXJpdHkgYW5kIEFsZ2FsIEJsb29tczogV2F0ZXIgY2xhcml0eSBtYXkgZGVjcmVhc2Ugc2xpZ2h0bHkgZHVlIHRvIHRoZSBpbml0aWFsIGFsZ2FsIGJsb29tcy4gSG93ZXZlciwgdGhpcyBwaGFzZSBpcyBvZnRlbiBzdGlsbCBjaGFyYWN0ZXJpemVkIGJ5IHJlbGF0aXZlbHkgZ29vZCB3YXRlciBxdWFsaXR5LiDigKIgQmlvZGl2ZXJzaXR5OiBUaGUgZWNvc3lzdGVtIG1pZ2h0IHN0aWxsIHN1cHBvcnQgYSBkaXZlcnNlIHJhbmdlIG9mIGFxdWF0aWMgb3JnYW5pc21zLCBpbmNsdWRpbmcgZmlzaCBhbmQgaW52ZXJ0ZWJyYXRlcy4gMy4xLjQuMiBQaGFzZSAyOiBNZXNvdHJvcGhpYyB0byBldXRyb3BoaWMgdHJhbnNpdGlvbiAxLiBOdXRyaWVudCBJbWJhbGFuY2Ug4oCiIFBob3NwaG9ydXMgdnMuIE5pdHJvZ2VuOiBBcyBhZ3JpY3VsdHVyYWwgcHJhY3RpY2VzIGludGVuc2lmeSBhbmQgdXJiYW5pemF0aW9uIGluY3JlYXNlcywgcGhvc3Bob3J1cyBpbnB1dHMgKGUuZy4sIGZyb20gZmVydGlsaXplcnMgYW5kIGRldGVyZ2VudHMpIG9mdGVuIGV4Y2VlZCB0aG9zZSBvZiBuaXRyb2dlbi4gVGhpcyBpbWJhbGFuY2UgY2FuIGRyaXZlIHNpZ25pZmljYW50IGNoYW5nZXMgaW4gd2F0ZXIgY2hlbWlzdHJ5IGFuZCBlY29zeXN0ZW0gZHluYW1pY3MuIDIuIEFsZ2FsIEJsb29tIEludGVuc2lmaWNhdGlvbiDigKIgQ3lhbm9iYWN0ZXJpYSBEb21pbmFuY2U6IEV4Y2VzcyBwaG9zcGhvcnVzIGNhbiBsZWFkIHRvIHRoZSBkb21pbmFuY2Ugb2YgY3lhbm9iYWN0ZXJpYSAoYmx1ZS1ncmVlbiBhbGdhZSksIHdoaWNoIGNhbiBvdXRjb21wZXRlIG90aGVyIGFsZ2FlIGFuZCBhcXVhdGljIHBsYW50cy4gVGhlc2UgYmxvb21zIGNhbiBiZSB0b3hpYyBhbmQgaGFybWZ1bCB0byBhcXVhdGljIGxpZmUuIOKAoiBIeXBveGlhOiBUaGUgZGVjb21wb3NpdGlvbiBvZiBsYXJnZSBhbGdhbCBibG9vbXMgY29uc3VtZXMgb3h5Z2VuLCBsZWFkaW5nIHRvIGh5cG94aWMgKGxvdyBveHlnZW4pIG9yIGFub3hpYyAobm8gb3h5Z2VuKSBjb25kaXRpb25zLCBwYXJ0aWN1bGFybHkgaW4gZGVlcGVyIHdhdGVycy4gMy4gV2F0ZXIgUXVhbGl0eSBEZXRlcmlvcmF0aW9uIOKAoiBEZWNyZWFzZWQgQ2xhcml0eTogV2F0ZXIgY2xhcml0eSBzaWduaWZpY2FudGx5IGRlY3JlYXNlcyBkdWUgdG8gZGVuc2UgYWxnYWwgZ3Jvd3RoLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo2Ny4yNjEwMDE1ODY5MTQwNiwibGVmdCI6MTUuNTgzNDU4OTAwNDUxNjYsIndpZHRoIjoyMC40NjA2MTg5NzI3NzgzMiwiaGVpZ2h0IjoxLjkwNTE0ODYyNTM3Mzg0MDN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NzAuNTEzMDMxMDA1ODU5MzgsImxlZnQiOjE3LjM5ODIxNjI0NzU1ODU5NCwid2lkdGgiOjY2LjQwMzU0OTE5NDMzNTk0LCJoZWlnaHQiOjEuOTA1MTU3MDg5MjMzMzk4NH19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo3Mi4xMzkwNDU3MTUzMzIwMywibGVmdCI6MTkuMDg4MzU2MDE4MDY2NDA2LCJ3aWR0aCI6NjUuNzg4NTIwODEyOTg4MjgsImhlaWdodCI6My41Mjg0NjA1MDI2MjQ1MTE3fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjc3LjAxNzA5NzQ3MzE0NDUzLCJsZWZ0IjoxNy4zOTgyMTYyNDc1NTg1OTQsIndpZHRoIjo2Mi4wOTQ1NzAxNTk5MTIxMSwiaGVpZ2h0IjoxLjkwNTE1NzA4OTIzMzM5ODR9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NzguNjQzMTEyMTgyNjE3MTksImxlZnQiOjE5LjA4ODM1NjAxODA2NjQwNiwid2lkdGgiOjM2LjcyNTMxMTI3OTI5Njg3NSwiaGVpZ2h0IjoxLjkwMjQ0NDI0MzQzMTA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6ODEuODk1MTMzOTcyMTY3OTcsImxlZnQiOjEzLjI0Njg1OTU1MDQ3NjA3NCwid2lkdGgiOjQwLjM5MTM5NTU2ODg0NzY1NiwiaGVpZ2h0IjoxLjkwNTE1NzA4OTIzMzM5ODR9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6ODUuMTQ3MTYzMzkxMTEzMjgsImxlZnQiOjE1LjU4MzQ1ODkwMDQ1MTY2LCJ3aWR0aCI6MTcuNjAyMTc4NTczNjA4NCwiaGVpZ2h0IjoxLjkwNTE1NzA4OTIzMzM5ODR9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6ODguMzk5MTkyODEwMDU4NiwibGVmdCI6MTcuMzk4MjE2MjQ3NTU4NTk0LCJ3aWR0aCI6NjUuMDA3MzQ3MTA2OTMzNiwiaGVpZ2h0IjoxLjkwNTE1NzA4OTIzMzM5ODR9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6OTAuMDI1MjA3NTE5NTMxMjUsImxlZnQiOjE5LjA4ODM1NjAxODA2NjQwNiwid2lkdGgiOjYxLjA1MTA1MjA5MzUwNTg2LCJoZWlnaHQiOjEuOTAyNDQ0MjQzNDMxMDkxM319LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjoxMC4zNTA0MTcxMzcxNDU5OTYsImxlZnQiOjIwLjg0MDgwNTA1MzcxMDkzOCwid2lkdGgiOjY2LjI3NzA1MzgzMzAwNzgxLCJoZWlnaHQiOjMuNTI4NDU1MjU3NDE1NzcxNX19LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjoxNS40OTk0Njc4NDk3MzE0NDUsImxlZnQiOjE3LjMzNTkwNjk4MjQyMTg3NSwid2lkdGgiOjIzLjUxOTYxNTE3MzMzOTg0NCwiaGVpZ2h0IjoxLjkwNTE0ODYyNTM3Mzg0MDN9fSx7InBhZ2VOdW1iZXIiOjEzLCJiYm94Ijp7InRvcCI6MTguNzUxNTAxMDgzMzc0MDIzLCJsZWZ0IjoxOS4xNTA2NjUyODMyMDMxMjUsIndpZHRoIjo2NC40Mzg3NjY0Nzk0OTIxOSwiaGVpZ2h0IjoxLjkwNTE0ODYyNTM3Mzg0MDN9fSx7InBhZ2VOdW1iZXIiOjEzLCJiYm94Ijp7InRvcCI6MjAuMzc3NTE3NzAwMTk1MzEyLCJsZWZ0IjoyMC44NDA4MDUwNTM3MTA5MzgsIndpZHRoIjo2MS4wNzQ0MjQ3NDM2NTIzNDQsImhlaWdodCI6My41Mjg0NTQ1NDIxNjAwMzR9fSx7InBhZ2VOdW1iZXIiOjEzLCJiYm94Ijp7InRvcCI6MjUuMjU1NTY3NTUwNjU5MTgsImxlZnQiOjE5LjE1MDY2NTI4MzIwMzEyNSwid2lkdGgiOjY4LjAyMzU1OTU3MDMxMjUsImhlaWdodCI6MS45MDUxNDg2MjUzNzM4NDAzfX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjI2Ljg4MTU4MjI2MDEzMTgzNiwibGVmdCI6MjAuODQwODA1MDUzNzEwOTM4LCJ3aWR0aCI6NjYuMDc2MjkzOTQ1MzEyNSwiaGVpZ2h0IjozLjUyODQ3MDk5MzA0MTk5Mn19LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjozMi4wMzA2NTEwOTI1MjkzLCJsZWZ0IjoxNy4zMzU5MDY5ODI0MjE4NzUsIndpZHRoIjoyNC42Nzc5NzY2MDgyNzYzNjcsImhlaWdodCI6MS45MDUxNDg2MjUzNzM4NDAzfX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjM1LjI4MjY4NDMyNjE3MTg3NSwibGVmdCI6MTkuMTUwNjY1MjgzMjAzMTI1LCJ3aWR0aCI6NjIuNDE5Nzg4MzYwNTk1NywiaGVpZ2h0IjoxLjkwNTE0ODYyNTM3Mzg0MDN9fSx7InBhZ2VOdW1iZXIiOjEzLCJiYm94Ijp7InRvcCI6MzYuOTA4Njk5MDM1NjQ0NTMsImxlZnQiOjIwLjg0MDgwNTA1MzcxMDkzOCwid2lkdGgiOjYuNjMzOTkzMTQ4ODAzNzExLCJoZWlnaHQiOjEuOTAyNDQwMTkwMzE1MjQ2Nn19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEzIn19LHsib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRpaW5hIiwiZmFtaWx5IjoiTsO1Z2VzIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hc2ltZSIsImZhbWlseSI6IkphbmF0aWFuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlJlZXQiLCJmYW1pbHkiOiJMYXVnYXN0ZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQZWV0ZXIiLCJmYW1pbHkiOiJOw7VnZXMifX1dLCJpZHMiOnsiaXNzbiI6WyIyMzUxLTk4OTQiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMTYvai5nZWNjby4yMDIwLmUwMTM2OSIsIm1hZyI6IjMxMDU0NDYwOTAiLCJvcGVuYWxleCI6IlczMTA1NDQ2MDkwIiwiZXhhIjoiaHR0cHM6Ly93d3cuc2NpZW5jZWRpcmVjdC5jb20vc2NpZW5jZS9hcnRpY2xlL3BpaS9TMjM1MTk4OTQyMDMwOTEwMCJ9LCJ0aXRsZSI6IlBvc3Qtc292aWV0IGNoYW5nZXMgaW4gbml0cm9nZW4gYW5kIHBob3NwaG9ydXMgc3RvaWNoaW9tZXRyeSBpbiB0d28gbGFyZ2Ugbm9uLXN0cmF0aWZpZWQgbGFrZXMgYW5kIHRoZSBpbXBhY3Qgb24gcGh5dG9wbGFua3RvbiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2ouZ2VjY28uMjAyMC5lMDEzNjkiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2ouZ2VjY28uMjAyMC5lMDEzNjkiLCJpc3N1ZWREYXRlIjoiMjAyMC0xMS0yMCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTEiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6Ikdsb2JhbCBFY29sb2d5IGFuZCBDb25zZXJ2YXRpb24iLCJ2b2x1bWUiOjI0LCJpbXBhY3RGYWN0b3IiOiIzLjU5In0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkVsc2V2aWVyIEJWIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo3MSwiY2l0ZWRCeUNvdW50IjoxNH19LCJxdW90ZSI6IlJhbmRvbSBGb3Jlc3QgYW5hbHlzaXMgaW5kaWNhdGVkIGFuIGltcG9ydGFudCByb2xlIG9mIGxpZ2h0IGxpbWl0YXRpb24gaW4gYm90aCBsYWtlcy4gQ29uc3RhbnRseSBsb3dlciBsZXZlbHMgb2YgTiBpbiB0aGUgZGVlcGVyIExha2UgUGVpcHNpIGZhdm91cmVkIE4yLWZpeGluZyBzcGVjaWVzLCB3aGljaCwgYXMgYSBwYXJhZG94LCBiZWNhbWUgUC1saW1pdGVkLiBBcyBjbGltYXRlIHdhcm1pbmcgcmVpbmZvcmNlcyBldXRyb3BoaWNhdGlvbiBwaGVub21lbmEgaW4gbGFrZXMgYnkgaW5jcmVhc2luZyBpbnRlcm5hbCBudXRyaWVudCBsb2FkaW5nIGFuZCBmYXZvdXJpbmcgYmxvb20tZm9ybWluZyBjeWFub2JhY3RlcmlhLCBtb3JlIHN0cmluZ2VudCBtZWFzdXJlcyB3b3VsZCBiZSBuZWVkZWQgdG8gZnVydGhlciBsaW1pdCBudXRyaWVudCBsb2FkcyAoZXNwZWNpYWxseSB0aGF0IG9mIFApIHRvIGxha2VzIHRocm91Z2ggaW1wcm92ZWQgd2FzdGV3YXRlciB0cmVhdG1lbnQgYW5kIGluY3JlYXNlZCBlZmZpY2llbmN5IG9mIGZlcnRpbGlzZXIgYXBwbGljYXRpb24uIExvbmctdGVybSBjaGFuZ2VzIGluIG51dHJpZW50IGxvYWRpbmcgYW5kIHN0b2ljaGlvbWV0cnkgaW4gd2F0ZXIgYm9kaWVzIHJlZmxlY3QgY2hhbmdlcyBpbiBhZ3JpY3VsdHVyYWwgcHJhY3RpY2VzIGFuZCB3YXN0ZXdhdGVyIHRyZWF0bWVudC4ifV0%3D. Short hydraulic retention times in shallow urban lakes exacerbate this, preventing dilution and amplifying bloom intensity (Gobler et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkNocmlzdG9waGVyIEouIiwiZmFtaWx5IjoiR29ibGVyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlJ1dGggVy4iLCJmYW1pbHkiOiJEcmlua3dhdGVyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkEuIFcuIiwiZmFtaWx5IjoiQW50aG9ueSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJKZW5uaWZlciBBLiIsImZhbWlseSI6IkdvbGVza2kifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW5uIE1hcmllIEUuIiwiZmFtaWx5IjoiRmFtdWxhcm8tUGVjb3JhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1pY2hhZWwgQi4iLCJmYW1pbHkiOiJXYWxsYWNlIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5vcmEgUi4gVy4iLCJmYW1pbHkiOiJTdHJhcXVhZGluZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJSb25vam95IiwiZmFtaWx5IjoiSGVtIn19XSwiaWRzIjp7Imlzc24iOlsiMTY2NC0zMDJYIl0sImlzYm4iOltdLCJkb2kiOiIxMC4zMzg5L2ZtaWNiLjIwMjQuMTQ2NDY4NiIsInBtaWQiOiIzOTU2ODk5NyIsIm9wZW5hbGV4IjoiVzQ0MDQxMjA5NTIiLCJleGEiOiJodHRwczovL3d3dy5mcm9udGllcnNpbi5vcmcvam91cm5hbHMvbWljcm9iaW9sb2d5L2FydGljbGVzLzEwLjMzODkvZm1pY2IuMjAyNC4xNDY0Njg2L2Z1bGwifSwidGl0bGUiOiJTZXdhZ2UtYW5kIGZlcnRpbGl6ZXItZGVyaXZlZCBudXRyaWVudHMgYWx0ZXIgdGhlIGludGVuc2l0eSwgZGl2ZXJzaXR5LCBhbmQgdG94aWNpdHkgb2YgaGFybWZ1bCBjeWFub2JhY3RlcmlhbCBibG9vbXMgaW4gZXV0cm9waGljIGxha2VzIiwiYWJzdHJhY3QiOiJDeWFub2JhY3RlcmlhbCBoYXJtZnVsIGFsZ2FsIGJsb29tcyAoQ0hBQnMpIGFyZSBwcm9tb3RlZCBieSBleGNlc3NpdmUgbnV0cmllbnQgbG9hZGluZyBhbmQsIHdoaWxlIGZlcnRpbGl6ZXJzIGFuZCBzZXdhZ2UgYXJlIHRoZSBtb3N0IHByZXZhbGVudCBleHRlcm5hbCBudXRyaWVudCBzb3VyY2VzIGluIG1vc3Qgd2F0ZXJzaGVkcywgdGhlIGRpZmZlcmVudGlhbCBlZmZlY3RzIG9mIHRoZXNlIG51dHJpZW50IHNvdXJjZXMgb24gQ0hBQnMgYXJlIHVua25vd24uIEhlcmUsIHdlIHRyYWNrZWQgQ0hBQnMgYW5kIHBlcmZvcm1lZCBleHBlcmltZW50cyBpbiBmaXZlIGRpc3RpbmN0IGxha2VzIGFjcm9zcyB0aGUgTm9ydGhlcm4gVVMgaW5jbHVkaW5nIExha2UgRXJpZS4gRmVydGlsaXplcnMgd2l0aCBhbW1vbml1bSBhbmQgb3J0aG9waG9zcGhhdGUsIG1lbWJyYW5lICgwLjIgzrxtKS1maWx0ZXJlZCBzZXdhZ2UgKGRvbWluYXRlZCBieSByZWR1Y2VkIGZvcm1zIG9mIG5pdHJvZ2VuKSBzYW5kLWFuZCBtZW1icmFuZS1maWx0ZXJlZCBzZXdhZ2UgKGRvbWluYXRlZCBieSBuaXRyYXRlKSwgYW5kIGFuIGlub3JnYW5pYyBudXRyaWVudCBzb2x1dGlvbiBvZiBhbW1vbml1bSBhbmQgb3J0aG9waG9zcGhhdGUgd2VyZSB1c2VkIGFzIGV4cGVyaW1lbnRhbCBudXRyaWVudCBzb3VyY2VzIGZvciBDSEFCcyBhdCBOLWVxdWl2YWxlbnQsIGVudmlyb25tZW50YWxseSByZWFsaXN0aWMgY29uY2VudHJhdGlvbnMuIFBoeXRvcGxhbmt0b24gY29tbXVuaXRpZXMgd2VyZSBldmFsdWF0ZWQgZmx1b3JvbWV0cmljYWxseSwgbWljcm9zY29waWNhbGx5LCBhbmQgdmlhIGhpZ2ggdGhyb3VnaHB1dCBzZXF1ZW5jaW5nIG9mIHRoZSAxNlMgclJOQSBnZW5lLCBhbmQgbGV2ZWxzIG9mIG1pY3JvY3lzdGluIGFuZCB0aGUgzrQgMTUgTiBjb250ZW50IG9mIHBhcnRpY3VsYXRlIG9yZ2FuaWMgbml0cm9nZW4gKM60UE8gMTUgTikgd2VyZSBxdWFudGlmaWVkLiBGZXJ0aWxpemVyIGFuZCBib3RoIHNvdXJjZXMgb2Ygd2FzdGV3YXRlciBpbmNyZWFzZWQgdGhlIGFidW5kYW5jZSBvZiBjeWFub2JhY3RlcmlhIGluIGFsbCBleHBlcmltZW50cyBhY3Jvc3MgYWxsIGZpdmUgbGFrZXMgKCBwICZhbXA7bHQ7IDAuMDUgZm9yIGFsbCkgd2hlcmVhcyBlZmZlY3RzIG9uIGV1a2FyeW90aWMgcGh5dG9wbGFua3RvbiB3ZXJlIGxpbWl0ZWQuIFNhbmQtZmlsdGVyZWQgc2V3YWdlIGNvbnRhaW5lZCBsZXNzIFAsIG9yZ2FuaWMgbWF0dGVyLCBhbmQgYW1tb25pdW0gYnV0IG1vcmUgbml0cmF0ZSBhbmQgaGFkIGEgMjUlIGxlc3MgcG90ZW50IHN0aW11bGF0b3J5IGVmZmVjdCBvbiBjeWFub2JhY3RlcmlhIHRoYW4gbWVtYnJhbmUtZmlsdGVyZWQgc2V3YWdlLCBzdWdnZXN0aW5nIG5pdHJpZmljYXRpb24gbWF5IHBsYXkgYSByb2xlIGluIHJlZHVjaW5nIENIQUJzLiBGZXJ0aWxpemVyIGluY3JlYXNlZCBtaWNyb2N5c3RpbiBsZXZlbHMgYW5kIGRlY3JlYXNlZCB0aGUgzrRQTyAxNSBOIHdoZXJlYXMgd2FzdGV3YXRlciBpbmNyZWFzZWQgzrRQTyAxNSBOICggcCAmYW1wO2x0OyAwLjA1IGZvciBhbGwpLiBNaWNyb2N5c3RpcyB3YXMgdGhlIGdlbnVzIG1vc3QgY29uc2lzdGVudGx5IHByb21vdGVkIGJ5IG51dHJpZW50IHNvdXJjZXMgKCBwICZhbXA7bHQ7IDAuMDUgaW4gYWxsIGV4cGVyaW1lbnRzKSwgZm9sbG93ZWQgYnkgQ3lhbm9iaXVtICggcCAmYW1wO2x0OyAwLjA1IGluIDUwJSBvZiBleHBlcmltZW50cyksIHdpdGggaW5jcmVhc2VzIGluIE1pY3JvY3lzdGlzIGJpb21hc3MgY29uc2lzdGVudGx5IGVsaWNpdGVkIGJ5IG1lbWJyYW5lLWZpbHRlcmVkIHdhc3Rld2F0ZXIuIENvbGxlY3RpdmVseSwgcmVzdWx0cyBkZW1vbnN0cmF0ZSB0aGF0IGRpZmZlcmluZyB0eXBlcyBvZiBzZXdhZ2UgZGlzY2hhcmdlIGFuZCBmZXJ0aWxpemVycyBjYW4gcHJvbW90ZSBDSEFCIGludGVuc2l0eSBhbmQgdG94aWNpdHksIHdoaWxlIGNvbmN1cnJlbnRseSBhbHRlcmluZyBDSEFCIGRpdmVyc2l0eSBhbmQgzrRQTyAxNSBOLiBXaGlsZSBtZW1icmFuZS1maWx0ZXJlZCBzZXdhZ2UgY29uc2lzdGVudGx5IGZhdm9yZWQgTWljcm9jeXN0aXMgLCB0aGUgZGlzY2hhcmdlIG9mIHNld2FnZSB0aHJvdWdoIHNhbmRzIG11dGVkIGJsb29tIGludGVuc2l0eSBzdWdnZXN0aW5nIHNhbmQtYmVkcyBtYXkgcmVwcmVzZW50IGEgdG9vbCB0byByZW1vdmUga2V5IG51dHJpZW50cyBhbmQgcGFydGlhbGx5IG1pdGlnYXRlIENIQUJzLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzg5L2ZtaWNiLjIwMjQuMTQ2NDY4NiIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cuZnJvbnRpZXJzaW4ub3JnL2pvdXJuYWxzL21pY3JvYmlvbG9neS9hcnRpY2xlcy8xMC4zMzg5L2ZtaWNiLjIwMjQuMTQ2NDY4Ni9wZGYiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMS0wNiIsImFjY2Vzc2VkRGF0ZSI6IjIwMjYtMDIiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZyb250aWVycyBpbiBNaWNyb2Jpb2xvZ3kiLCJ2b2x1bWUiOjE1LCJpbXBhY3RGYWN0b3IiOiIzLjk5In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTQ2NDY4NiwibGFzdFBhZ2UiOjE0NjQ2ODZ9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRnJvbnRpZXJzIE1lZGlhIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo3MywiY2l0ZWRCeUNvdW50Ijo5fX0sInF1b3RlIjoiZGVyaXZlZCBudXRyaWVudHMgYWx0ZXIgdGhlIGludGVuc2l0eSwgZGl2ZXJzaXR5LCBhbmQgdG94aWNpdHkgb2YgaGFybWZ1bCBjeWFub2JhY3RlcmlhbCBibG9vbXMgLi4uIEN5YW5vYmFjdGVyaWFsIGhhcm1mdWwgYWxnYWwgYmxvb21zIChDSEFCcykgYXJlIHByb21vdGVkIGJ5IGV4Y2Vzc2l2ZSBudXRyaWVudCBsb2FkaW5nIGFuZCwgd2hpbGUgZmVydGlsaXplcnMgYW5kIHNld2FnZSBhcmUgdGhlIG1vc3QgcHJldmFsZW50IGV4dGVybmFsIG51dHJpZW50IHNvdXJjZXMgaW4gbW9zdCB3YXRlcnNoZWRzLCB0aGUgZGlmZmVyZW50aWFsIGVmZmVjdHMgb2YgdGhlc2UgbnV0cmllbnQgLi4uIG9yZ2FuaWMgbml0cm9nZW4gKM60UE8xNU4pIHdlcmUgcXVhbnRpZmllZC4gRmVydGlsaXplciBhbmQgYm90aCBzb3VyY2VzIG9mIHdhc3Rld2F0ZXIgaW5jcmVhc2VkIHRoZSBhYnVuZGFuY2Ugb2YgLi4uIHBoeXRvcGxhbmt0b24gd2VyZSBsaW1pdGVkLiBTYW5kLWZpbHRlcmVkIHNld2FnZSBjb250YWluZWQgbGVzcyBQLCBvcmdhbmljIG1hdHRlciwgYW5kIGFtbW9uaXVtIGJ1dCBtb3JlIG5pdHJhdGUgYW5kIC4uLiBkaWZmZXJpbmcgdHlwZXMgb2Ygc2V3YWdlIGRpc2NoYXJnZSBhbmQgZmVydGlsaXplcnMgY2FuIHByb21vdGUgQ0hBQiBpbnRlbnNpdHkgYW5kIHRveGljaXR5LCB3aGlsZSBjb25jdXJyZW50bHkgLi4uIHNld2FnZSB0aHJvdWdoIHNhbmRzIG11dGVkIGJsb29tIGludGVuc2l0eSBzdWdnZXN0aW5nIHNhbmQtYmVkcyBtYXkgcmVwcmVzZW50IGEgdG9vbCB0byByZW1vdmUga2V5IG51dHJpZW50cyBhbmQgLi4uICggV2F0c29uIGV0IGFsLiwgMjAxNiA7IEJ1bGxlcmphaG4gZXQgYWwuLCAyMDE2IDsgSHVpc21hbiBldCBhbC4sIDIwMTggKSBhbmQgZXhjZXNzaXZlIG51dHJpZW50IGxvYWRpbmcgaXMgdGhlIHByb2Nlc3MgbW9zdCBjb21tb25seSBjaXRlZCBhcyB0aGUgZW52aXJvbm1lbnRhbCBmYWN0b3IgcHJvbW90aW5nIENIQUJzICgifV0%3D.

Figure 4: Untreated Urban Sewage

Industrial discharges compound the crisis, introducing not only macronutrients but also recalcitrant organics, heavy metals, and xenobiotics that synergize with eutrophication. Textile, pharmaceutical, and electroplating industries prevalent in peri-urban clusters release effluents laden with ammonium (from dyes) and phosphates (from processing aids), at concentrations 5-10 times permissible limits, directly fueling Microcystis and Planktothrix proliferations indicators of nitrogen over-enrichment (Çelekli̇, 2024; Paerl, 2018)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFidXplciIsImZhbWlseSI6IsOHZWxla2xpzIcifX1dLCJpZHMiOnsiaXNzbiI6W10sImlzYm4iOltdLCJkb2kiOiIxMC4yMTIwMy9ycy4zLnJzLTUwNzgxNzYvdjEiLCJvcGVuYWxleCI6Ilc0NDA0NTI5NDE4In0sInRpdGxlIjoiVGhlIGZpcnN0IGVjby1hc3Nlc3NtZW50IG9mIHBoeXRvcGxhbmt0b24gYXNzZW1ibGFnZXMgYXNzb2NpYXRlZCB3aXRoIGVudmlyb25tZW50YWwgY29uZGl0aW9ucyBvZiBkaWZmZXJlbnQgbGFrZXMgaW4gdGhlIFdlc3Rlcm4gQmxhY2sgU2VhIGJhc2luIG9mIFTDvHJraXllIiwiYWJzdHJhY3QiOiI8dGl0bGU%2BQWJzdHJhY3Q8L3RpdGxlPiBMYWtlcyBwbGF5IGEgdml0YWwgcm9sZSBpbiBzdXBwb3J0aW5nIGJpb2RpdmVyc2l0eSwgcHJvdmlkaW5nIHdhdGVyIHJlc291cmNlcywgcmVndWxhdGluZyBjbGltYXRlLCBjeWNsaW5nIG51dHJpZW50cywgYW5kIG9mZmVyaW5nIHJlY3JlYXRpb25hbCBvcHBvcnR1bml0aWVzLiBEZXNwaXRlIHRoZWlyIGltcG9ydGFuY2UgZm9yIGVudmlyb25tZW50YWwgaGVhbHRoIGFuZCBodW1hbiB3ZWxsLWJlaW5nLCBsYWtlcyBmYWNlIHNpZ25pZmljYW50IHByZXNzdXJlcyBpbiB0aGUgQW50aHJvcG9jZW5lIGVyYS4gVGhlIHByZXNlbnQgd29yayBzZWVrcyB0byBhc3Nlc3MgdGhlIHNwZWNpZXMtZW52aXJvbm1lbnQgaW50ZXJhY3Rpb25zIGFuZCB0aGUgZWNvbG9naWNhbCBzdGF0dXMgb2Ygc2l4IGxha2VzIGluIHRoZSBXZXN0ZXJuIEJsYWNrIFNlYSBiYXNpbiBvZiBUw7xya2l5ZSB1dGlsaXppbmcgcGh5dG9wbGFua3RvbiBtZXRyaWNzIGR1cmluZyB3ZXQgYW5kIGRyeSBwZXJpb2RzLiBDYW5vbmljYWwgY29ycmVzcG9uZGVuY2UgYW5hbHlzaXMgcmV2ZWFsZWQgYSBzaWduaWZpY2FudCBjb3JyZWxhdGlvbiBlcXVhbCB0byA5OC41JSBiZXR3ZWVuIHBoeXRvcGxhbmt0b24gc3BlY2llcyBhbmQgZW52aXJvbm1lbnRhbCBzdHJlc3NvcnMgZHVyaW5nIHR3byBoeWRyb2xvZ2ljYWwgcGVyaW9kcy4gRWxlY3RyaWNhbCBjb25kdWN0aXZpdHkgKEVDKSwgcEgsIHRvdGFsIG9yZ2FuaWMgY2FyYm9uIChUT0MpLCBhbmQgdGVtcGVyYXR1cmUgd2VyZSB0aGUgbW9zdCBpbmZsdWVudGlhbCBlbnZpcm9ubWVudGFsIGZhY3RvcnMgYWZmZWN0aW5nIHBoeXRvcGxhbmt0b24gZGlzdHJpYnV0aW9uIGluIGxha2VzIChwPTAuMDAyKS4gTGFrZSBTYXLEsWt1bSwgYSBicmFja2lzaCBoYWJpdGF0LCB3YXMgdW5kZXIgaGlnaCBFQyBhbmQgcEggcHJlc3N1cmUgYW5kIGlzIGNoYXJhY3Rlcml6ZWQgYnkgcG9sbHV0aW9uLXRvbGVyYW50IHNwZWNpZXMuIExha2UgWWVuacOnYcSfYSBhc3NvY2lhdGVkIHdpdGggVE9DIGlzIGNoYXJhY3Rlcml6ZWQgYnkgc29tZSBzcGVjaWVzLCBzdWNoIGFzIDxpdGFsaWM%2BQW5hYmFlbm9wc2lzPC9pdGFsaWM%2BIDxpdGFsaWM%2BbWlsbGVyaTwvaXRhbGljPiwgPGl0YWxpYz5DaHJvb2NvY2N1czwvaXRhbGljPiA8aXRhbGljPnR1cmdpZHVzPC9pdGFsaWM%2BLDxpdGFsaWM%2BIFBzZXVkb3NjaHJvZWRlcmlhPC9pdGFsaWM%2BIDxpdGFsaWM%2Bcm9idXN0YTwvaXRhbGljPiwgPGl0YWxpYz5BcGhhbm9jYXBzYTwvaXRhbGljPiBzcC4sIDxpdGFsaWM%2BTWVyaXNtb3BlZGlhPC9pdGFsaWM%2BIDxpdGFsaWM%2BZ2xhdWNhPC9pdGFsaWM%2BLCA8aXRhbGljPk1pY3JhY3Rpbml1bTwvaXRhbGljPiA8aXRhbGljPnF1YWRyaXNldHVtPC9pdGFsaWM%2BLCBhbmQ8aXRhbGljPiBNaWNyb2N5c3RpczwvaXRhbGljPiA8aXRhbGljPmFlcnVnaW5vc2E8L2l0YWxpYz4uIExha2UgQWJhbnQgaXMgbG9jYXRlZCBvbiB0aGUgb3Bwb3NpdGUgc2lkZSBvZiBFQywgVE9DLCBhbmQgdGVtcGVyYXR1cmUsIHdoaWNoIHdhcyBhc3NvY2lhdGVkIHdpdGggc29tZSBzcGVjaWVzIHN1Y2ggYXMgPGl0YWxpYz5DeW1iZWxsYSBhZmZpbmlzPC9pdGFsaWM%2BLCA8aXRhbGljPkFjaG5hbnRoaWRpdW08L2l0YWxpYz4gPGl0YWxpYz5taW51dGlzc2ltdW08L2l0YWxpYz4sIDxpdGFsaWM%2BRW5jeW9uZW1hPC9pdGFsaWM%2BIDxpdGFsaWM%2BbWludXR1bTwvaXRhbGljPiwgPGl0YWxpYz5FLjwvaXRhbGljPiA8aXRhbGljPnNpbGVzaWFjdW08L2l0YWxpYz4sIGFuZDxpdGFsaWM%2BIERpbm9icnlvbjwvaXRhbGljPiA8aXRhbGljPmRpdmVyZ2VuczwvaXRhbGljPi4gUmVzdWx0cyBvZiB0aGUgcGh5dG8tYXNzZXNzbWVudCBkaXNwbGF5ZWQgdGhhdCB0aGUgZWNvbG9naWNhbCBzdGF0dXMgKEVTKSBvZiB0aGUgc2FtcGxpbmcgc3RhdGlvbnMgZHVyaW5nIHRoZSByYWlueSBhbmQgZHJ5IHBlcmlvZHMgdmFyaWVkIGZyb20gYmFkIHRvIGhpZ2guIFRoZSBtb2RpZmllZCBwaHl0b3BsYW5rdG9uIHRyb3BoaWMgaW5kZXggKE1QVEkpIGV4aGliaXRlZCB0aGF0IGEgbW9kZXJhdGUgRVMgd2FzIGZvdW5kIGluIExha2VzIFNhcsSxa3VtIGFuZCBZZW5pw6dhxJ9hLCB3aGlsZSBvdGhlcnMgaGFkIGEgZ29vZCBFUy4gVGhlIHByZXNlbnQgc3R1ZHkgY29u76yBcm1lZCB0aGF0IHBoeXRvcGxhbmt0b24gY29tbXVuaXRpZXMgYXJlIHN0cm9uZ2x5IGxpbmtlZCB0byB0aGUgZWNvbG9naWNhbCBzdGF0dXMgb2YgbGFrZXMgaW4gdGhlIFdlc3Rlcm4gQmxhY2sgU2VhIGJhc2luLCB3aGljaCBjb3VsZCBiZSBhc3Nlc3NlZCB1c2luZyB0aGUgTVBUSS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjEyMDMvcnMuMy5ycy01MDc4MTc2L3YxIiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5yZXNlYXJjaHNxdWFyZS5jb20vYXJ0aWNsZS9ycy01MDc4MTc2L2xhdGVzdC5wZGYiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMS0xOSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDgiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IlJlc2VhcmNoIFNxdWFyZSAoUmVzZWFyY2ggU3F1YXJlKSJ9LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJSZXNlYXJjaCBTcXVhcmUgKFVuaXRlZCBTdGF0ZXMpIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo1MiwiY2l0ZWRCeUNvdW50IjowfX0sInF1b3RlIjoiRGVzcGl0ZSB0aGVpciBlY29sb2dpY2FsIHNpZ25p77%2B9Y2FuY2UsIHRoZXNlIHdhdGVyIGJvZGllcyBhcmUgaW5jcmVhc2luZ2x5IHRocmVhdGVuZWQgYnkgYW50aHJvcG9nZW5pYyBhY3Rpdml0aWVzIChlLmcuLCB1cmJhbml6YXRpb24sIGFncmljdWx0dXJhbCBsYW5kIHVzZSwgd2FzdGV3YXRlciBkaXNjaGFyZ2UsIHJlZWQgY3V0dGluZywgc2FuZCByZW1vdmFsLCBldGMuKSBhbmQgY2xpbWF0ZSBjaGFuZ2UgKEjDpGRlciBldCBhbC4gMjAyMDsgw4dlbGVrbGkgZXQgYWwuIDIwMjQpLiBOdXRyaWVudCBlbnJpY2htZW50IGluIGxlbnRpYyBlY29zeXN0ZW1zIGltcGFjdHMgbm90IG9ubHkgdGhlIHBoeXNpY29jaGVtaWNhbCBmYWN0b3JzIG9mIGxha2VzIGJ1dCBhbHNvIG5lZ2F0aXZlbHkgYWZmZWN0cyBhcXVhdGljIGxpZmUsIGNhdXNpbmcgaGFybWZ1bCBhbGdhbCBibG9vbXMsIGV4Y2Vzc2l2ZSBncm93dGggb2YgbnVpc2FuY2UgbWFjcm9waHl0ZXMsIGFuZCBpbmNyZWFzZWQg77%2B9c2ggbW9ydGFsaXR5LCBlc3BlY2lhbGx5IHNlbnNpdGl2ZSBzcGVjaWVzIChTdXJlc2ggZXQgYWwuIDIwMjMpLiBJdCBhbHNvIGRpc3J1cHRzIHRoZSBoeWRyb21vcnBob2xvZ3kgYW5kIG92ZXJhbGwgZnVuY3Rpb25pbmcgb2YgdGhlc2UgZWNvc3lzdGVtcy4gRGVzcGl0ZSBnbG9iYWwgd2F0ZXIgc2hvcnRhZ2VzLCB0aGUgZ3Jvd2luZyBkZWdyYWRhdGlvbiBvZiBlY29zeXN0ZW1zIGFuZCB0aGUgb3ZlcnVzZSBvZiBmcmVzaHdhdGVyIHJlc291cmNlcyBoYXZlIHNoaWZ0ZWQgYXR0ZW50aW9uIGJhY2sgdG8gZnJlc2h3YXRlciBlY29zeXN0ZW1zIHRvIGJldHRlciBtYW5hZ2UgYW5kIHByZXNlcnZlIHdhdGVyIHJlc291cmNlcyBnbG9iYWxseSAow4dlbGVrbGkgZXQgYWwuIDIwMjE7IFN1cmVzaCBldCBhbC4gMjAyMykuIEFzc2Vzc2luZyB0aGUgZWNvbG9naWNhbCBzdGF0dXMgb2YgbGFrZXMgdGhyb3VnaCBwaHl0b3BsYW5rdG9uIG1ldHJpY3MgaXMgYSBjcnVjaWFsIGNvbXBvbmVudCBvZiBlbnZpcm9ubWVudGFsIG1vbml0b3JpbmcgYW5kIG1hbmFnZW1lbnQsIGFzIGl0IGFsaWducyB3aXRoIHRoZSBndWlkZWxpbmVzIHNldCBieSB0aGUgV2F0ZXIgRnJhbWV3b3JrIERpcmVjdGl2ZSAoRGlyZWN0aXZlIDIwMDApLiBQaHl0b3BsYW5rdG9uLCBhcyBwcmltYXJ5IHByb2R1Y2VycywgYXJlIHNlbnNpdGl2ZSBpbmRpY2F0b3JzIG9mIGNoYW5nZXMgaW4gd2F0ZXIgcXVhbGl0eSBhbmQgZWNvc3lzdGVtIGhlYWx0aCAoUmV5bm9sZHMgZXQgYWwuIDIwMDI7IMOHZWxla2xpIGV0IGFsLiAyMDIzYSkuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NjYuOTY0MjI1NzY5MDQyOTcsImxlZnQiOjQuNTM0MzEzNjc4NzQxNDU1LCJ3aWR0aCI6OTAuNjE0NzYxMzUyNTM5MDYsImhlaWdodCI6MS43NzU1Mjk2MjMwMzE2MTYyfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NjkuMjM2OTA3OTU4OTg0MzgsImxlZnQiOjQuNTM0MzEzNjc4NzQxNDU1LCJ3aWR0aCI6OTAuMDEwMzA3MzEyMDExNzIsImhlaWdodCI6MS43NzU1Mjk2MjMwMzE2MTYyfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NzEuNTA5NTkwMTQ4OTI1NzgsImxlZnQiOjQuNTM0MzEzNjc4NzQxNDU1LCJ3aWR0aCI6ODUuODEyMTc5NTY1NDI5NjksImhlaWdodCI6MS43NzU1Mjk2MjMwMzE2MTYyfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NzMuNzgyMjcyMzM4ODY3MTksImxlZnQiOjQuNTM0MzEzNjc4NzQxNDU1LCJ3aWR0aCI6ODYuMDcwMzEyNSwiaGVpZ2h0IjoxLjc3NTUyOTYyMzAzMTYxNjJ9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo3Ni4wNTQ5NTQ1Mjg4MDg2LCJsZWZ0Ijo0LjUzNDMxMzY3ODc0MTQ1NSwid2lkdGgiOjg5LjYyMzY2NDg1NTk1NzAzLCJoZWlnaHQiOjEuNzc1NTI5NjIzMDMxNjE2Mn19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjc4LjMyNzYyOTA4OTM1NTQ3LCJsZWZ0Ijo0LjUzNDMxMzY3ODc0MTQ1NSwid2lkdGgiOjg5Ljg1OTc1NjQ2OTcyNjU2LCJoZWlnaHQiOjEuNzc1NTI5NjIzMDMxNjE2Mn19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjgwLjYwMDMxODkwODY5MTQsImxlZnQiOjQuNTM0MzEzNjc4NzQxNDU1LCJ3aWR0aCI6ODkuNjQwODM4NjIzMDQ2ODgsImhlaWdodCI6MS43NzU1Mjk2MjMwMzE2MTYyfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6ODIuODcyOTkzNDY5MjM4MjgsImxlZnQiOjQuNTM0MzEzNjc4NzQxNDU1LCJ3aWR0aCI6OTAuODM2NjM5NDA0Mjk2ODgsImhlaWdodCI6NC4wNDgxOTQ4ODUyNTM5MDZ9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo4Ny40MTgzNDI1OTAzMzIwMywibGVmdCI6NC41MzQzMTM2Nzg3NDE0NTUsIndpZHRoIjozMy43OTA5MzU1MTYzNTc0MiwiaGVpZ2h0IjoxLjc3NTUyOTYyMzAzMTYxNjJ9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo5MS4yMDYxNDYyNDAyMzQzOCwibGVmdCI6NC41MzQzMTM2Nzg3NDE0NTUsIndpZHRoIjo4NC42NzUzNzY4OTIwODk4NCwiaGVpZ2h0IjoxLjc3NTUyOTYyMzAzMTYxNjJ9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo5My40Nzg4MjA4MDA3ODEyNSwibGVmdCI6NC41MzQzMTM2Nzg3NDE0NTUsIndpZHRoIjo4OS45ODM4NDg1NzE3NzczNCwiaGVpZ2h0IjoxLjc3NTUyOTYyMzAzMTYxNjJ9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo0Ljc0OTY0MzgwMjY0MjgyMiwibGVmdCI6NC41MzQzMTM2Nzg3NDE0NTUsIndpZHRoIjo4Ny45NTc4NjI4NTQwMDM5LCJoZWlnaHQiOjEuNzc1NTMwNTc2NzA1OTMyNn19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjcuMDIyMzA5MzAzMjgzNjkxLCJsZWZ0Ijo0LjUzNDMxMzY3ODc0MTQ1NSwid2lkdGgiOjkwLjY2NjUxMTUzNTY0NDUzLCJoZWlnaHQiOjEuNzc1NTMwNTc2NzA1OTMyNn19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjIifX0seyJvcmlnaW4iOjQsIndvcmsiOnsidHlwZSI6OCwic3VidHlwZSI6eyJvbmVvZktpbmQiOiJhcnRpY2xlVHlwZSIsImFydGljbGVUeXBlIjoxfSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSGFucyBXLiIsImZhbWlseSI6IlBhZXJsIn19XSwiaWRzIjp7Imlzc24iOltdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMTc2MTUveW41NS1iNDM4Iiwib3BlbmFsZXgiOiJXNDMwNzA2MzczOSJ9LCJ0aXRsZSI6Ik1pdGlnYXRpbmcgdG94aWMgcGxhbmt0b25pYyBjeWFub2JhY3RlcmlhbCBibG9vbXMgaW4gYXF1YXRpYyBlY29zeXN0ZW1zIGZhY2luZyBpbmNyZWFzaW5nIGFudGhyb3BvZ2VuaWMgYW5kIGNsaW1hdGljIHByZXNzdXJlcyIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xNzYxNS95bjU1LWI0MzgiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vY2RyLmxpYi51bmMuZWR1L2Rvd25sb2Fkcy93aDI0NzI5MGciLCJpc3N1ZWREYXRlIjoiMjAxOC0wMS0wMSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDYiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkNhcm9saW5hIERpZ2l0YWwgUmVwb3NpdG9yeSAoVW5pdmVyc2l0eSBvZiBOb3J0aCBDYXJvbGluYSBhdCBDaGFwZWwgSGlsbCkiLCJpbXBhY3RGYWN0b3IiOiIwLjEwIn0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlVuaXZlcnNpdHkgb2YgTm9ydGggQ2Fyb2xpbmEgYXQgQ2hhcGVsIEhpbGwifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjAsImNpdGVkQnlDb3VudCI6MH19LCJxdW90ZSI6IlRoaXMgbGVhZHMgIHRvIGNvbnRpbnVpbmcgZGVtYW5kIGZvciBOIHN1cHBsaWVzIHRvIHN1c3RhaW4gcHJpbWFyeSBwcm9kdWN0aW9uIGFuZCBwcm9tb3RlIGV1dHJvcGhpY2F0aW9uICBbMzgsMzldLCAoMikgYSBsZW5ndGh5IGxlZ2FjeSBvZiBQIGxvYWRpbmcgdGhhdCBpcyBzdG9yZWQgaW4gc2VkaW1lbnRzLCB3aGljaCBpcyByZWFkaWx5IHJlY3ljbGVkIGFuZCAgc3VwcG9ydHMgZXV0cm9waGljYXRpb24sICgzKSBOMiBmaXhhdGlvbiwgd2hpbGUgcHJvdmlkaW5nIOKAnG5ld%2BKAnSBOLCBmYWlscyB0byBtZWV0IGVjb3N5c3RlbS1zY2FsZSBOICByZXF1aXJlbWVudHMgWzQwLDQxXS4gRnJvbSBhIG51dHJpZW50IG1hbmFnZW1lbnQgcGVyc3BlY3RpdmUsIHRoaXMgbWVhbnMgdGhhdCBjb250aW51ZWQgZXh0ZXJuYWwgIGxvYWRpbmcgb2YgTiBwbGF5cyBhIGNyaXRpY2FsIHJvbGUgaW4gYWNjZWxlcmF0aW5nIGV1dHJvcGhpY2F0aW9uIGFuZCBwcm9tb3RpbmcgYWxnYWwgYmxvb21zIFszNl0uICBPbmUgdHJvdWJsaW5nIGluZGljYXRvciBvZiBOLWRyaXZlbiBldXRyb3BoaWNhdGlvbiBhbmQgQ3lhbm9IQUJzIGluIGZyZXNod2F0ZXIgc3lzdGVtcyBpcyAgdGhlIGdlb2dyYXBoaWMgZXhwYW5zaW9uIGFuZCBpbmNyZWFzZSBmcmVxdWVuY3kgb2YgdGhlIG5vbi1OMiBmaXhpbmcgdG94aW4tcHJvZHVjaW5nIGdlbnVzICBNaWNyb2N5c3RpcyBbNDJdLCBlc3BlY2lhbGx5IGluIGxhcmdlIGxha2VzIChlLmcuLCBMYWtlIEVyaWUsIENhbmFkYS1VU0EsIExha2UgS2FzdW1hZ3VyYSwgSmFwYW4sIExha2UgIE9rZWVjaG9iZWUsIEZsb3JpZGEsIFVTQSwgTGFrZSBUYWlodSwgQ2hpbmEpLiBUaGlzIGdlbnVzLCBhcyB3ZWxsIGFzIGFub3RoZXIgcGxhbmt0b25pYyBub24tTjIgIGZpeGluZyBDeWFub0hBQiBQbGFua3RvdGhyaXgsIGFuZCBzZXZlcmFsIGJlbnRoaWMgYW5hbG9ndWUgYmxvb20gdGF4YSAoZS5nLiwgbm9uLU4yIGZpeGluZyBMeW5nYnlhICBhbmQgT3NjaWxsYXRvcmlhIHNwcC4pLCBzZXJ2ZSBhcyBpbmRpY2F0b3JzIG9mIE4tb3Zlci1lbnJpY2htZW50IFszNiw0Ml0uIFRoZSBwcm9saWZlcmF0aW9uIG9mIHRoZXNlIG5vbi0gRmlndXJlIDUuIEdsb2JhbCBOIGFuZCBQLWJhc2VkIGZlcnRpbGl6ZXIgdXNlIGluIHJlbGF0aW9uIHRvIHRoZSB3b3JsZOKAmXMgaHVtYW4gcG9wdWxhdGlvbi4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo3NS40NjUxMjYwMzc1OTc2NiwibGVmdCI6MjAuMDU4Mjk0Mjk2MjY0NjUsIndpZHRoIjo2NC4wMzU2OTc5MzcwMTE3MiwiaGVpZ2h0IjoxLjA4ODQyMzYwOTczMzU4MTV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo3Ni45MDgzMDIzMDcxMjg5LCJsZWZ0IjoxNi44NDY5NTI0MzgzNTQ0OTIsIndpZHRoIjo2Ny4yMzg4NjEwODM5ODQzOCwiaGVpZ2h0IjoxLjA4ODQyMzYwOTczMzU4MTV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo3OC4zNTE0Nzg1NzY2NjAxNiwibGVmdCI6MTYuODQ2OTUyNDM4MzU0NDkyLCJ3aWR0aCI6NjcuMjQ3MDcwMzEyNSwiaGVpZ2h0IjoxLjA4ODQyMzYwOTczMzU4MTV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo3OS43OTQ2NTQ4NDYxOTE0LCJsZWZ0IjoxNi44NDY5NTI0MzgzNTQ0OTIsIndpZHRoIjo2Ny4yNTAwMjI4ODgxODM2LCJoZWlnaHQiOjEuMDg4NTEwNjMyNTE0OTUzNn19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjgxLjIzNzkyMjY2ODQ1NzAzLCJsZWZ0IjoxNi44NDcwNDAxNzYzOTE2LCJ3aWR0aCI6NjcuMjQwMDUxMjY5NTMxMjUsImhlaWdodCI6MS4wODg0MjM2MDk3MzM1ODE1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6ODIuNjgxMDk4OTM3OTg4MjgsImxlZnQiOjE2Ljg0NzA0MDE3NjM5MTYsIndpZHRoIjo2NC43MDg5MTU3MTA0NDkyMiwiaGVpZ2h0IjoxLjA4ODQyMzYwOTczMzU4MTV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo4NC4xMjQyODI4MzY5MTQwNiwibGVmdCI6MjAuMDU4MzgzOTQxNjUwMzksIndpZHRoIjo2NC4wMjcyMDY0MjA4OTg0NCwiaGVpZ2h0IjoxLjA4ODQyMzYwOTczMzU4MTV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo4NS41NjEwMjc1MjY4NTU0NywibGVmdCI6MTYuODQ3MDM2MzYxNjk0MzM2LCJ3aWR0aCI6NjcuMjMzMzY3OTE5OTIxODgsImhlaWdodCI6MS4wODg0NTk4NDkzNTc2MDV9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo4Ny4wMDQyNDE5NDMzNTkzOCwibGVmdCI6MTYuODQ3MDA3NzUxNDY0ODQ0LCJ3aWR0aCI6NjcuMjQ4MjIyMzUxMDc0MjIsImhlaWdodCI6MS4wODg0MjM2MDk3MzM1ODE1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6ODguNDQ3NDAyOTU0MTAxNTYsImxlZnQiOjE2Ljg0NzAwNzc1MTQ2NDg0NCwid2lkdGgiOjY3LjA4MzE1Mjc3MDk5NjEsImhlaWdodCI6MS4wODg0MjM2MDk3MzM1ODE1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6ODkuODkwNjAyMTExODE2NCwibGVmdCI6MTYuODQ3MDA3NzUxNDY0ODQ0LCJ3aWR0aCI6NjcuMjM0ODkzNzk4ODI4MTIsImhlaWdodCI6MS4wODg0MjM2MDk3MzM1ODE1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6OTEuMzMzNzc4MzgxMzQ3NjYsImxlZnQiOjE2Ljg0Njk4NDg2MzI4MTI1LCJ3aWR0aCI6NjYuODYyNDExNDk5MDIzNDQsImhlaWdodCI6MS4wODg0MjM2MDk3MzM1ODE1fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NTYuODE4MTY0ODI1NDM5NDUsImxlZnQiOjE2LjQyODUxNDQ4MDU5MDgyLCJ3aWR0aCI6Ni4zNTY0MTQzMTgwODQ3MTcsImhlaWdodCI6MS4zMTYzNzg0NzQyMzU1MzQ3fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6NTYuODA5NjM4OTc3MDUwNzgsImxlZnQiOjI0LjQ5NDM1MjM0MDY5ODI0Miwid2lkdGgiOjU5LjM0MTMwMDk2NDM1NTQ3LCJoZWlnaHQiOjEuMzA1NzMyMTMxMDA0MzMzNX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjYifX1d. Studies show sewage-derived nutrients heighten cyanotoxin production, with microcystin levels surging 2-5 fold in impacted systems, as organic nitrogen and phosphorus enhance both biomass and toxicity (Gobler et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkNocmlzdG9waGVyIEouIiwiZmFtaWx5IjoiR29ibGVyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlJ1dGggVy4iLCJmYW1pbHkiOiJEcmlua3dhdGVyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkEuIFcuIiwiZmFtaWx5IjoiQW50aG9ueSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJKZW5uaWZlciBBLiIsImZhbWlseSI6IkdvbGVza2kifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW5uIE1hcmllIEUuIiwiZmFtaWx5IjoiRmFtdWxhcm8tUGVjb3JhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1pY2hhZWwgQi4iLCJmYW1pbHkiOiJXYWxsYWNlIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5vcmEgUi4gVy4iLCJmYW1pbHkiOiJTdHJhcXVhZGluZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJSb25vam95IiwiZmFtaWx5IjoiSGVtIn19XSwiaWRzIjp7Imlzc24iOlsiMTY2NC0zMDJYIl0sImlzYm4iOltdLCJkb2kiOiIxMC4zMzg5L2ZtaWNiLjIwMjQuMTQ2NDY4NiIsInBtaWQiOiIzOTU2ODk5NyIsIm9wZW5hbGV4IjoiVzQ0MDQxMjA5NTIiLCJleGEiOiJodHRwczovL3d3dy5mcm9udGllcnNpbi5vcmcvam91cm5hbHMvbWljcm9iaW9sb2d5L2FydGljbGVzLzEwLjMzODkvZm1pY2IuMjAyNC4xNDY0Njg2L2Z1bGwifSwidGl0bGUiOiJTZXdhZ2UtYW5kIGZlcnRpbGl6ZXItZGVyaXZlZCBudXRyaWVudHMgYWx0ZXIgdGhlIGludGVuc2l0eSwgZGl2ZXJzaXR5LCBhbmQgdG94aWNpdHkgb2YgaGFybWZ1bCBjeWFub2JhY3RlcmlhbCBibG9vbXMgaW4gZXV0cm9waGljIGxha2VzIiwiYWJzdHJhY3QiOiJDeWFub2JhY3RlcmlhbCBoYXJtZnVsIGFsZ2FsIGJsb29tcyAoQ0hBQnMpIGFyZSBwcm9tb3RlZCBieSBleGNlc3NpdmUgbnV0cmllbnQgbG9hZGluZyBhbmQsIHdoaWxlIGZlcnRpbGl6ZXJzIGFuZCBzZXdhZ2UgYXJlIHRoZSBtb3N0IHByZXZhbGVudCBleHRlcm5hbCBudXRyaWVudCBzb3VyY2VzIGluIG1vc3Qgd2F0ZXJzaGVkcywgdGhlIGRpZmZlcmVudGlhbCBlZmZlY3RzIG9mIHRoZXNlIG51dHJpZW50IHNvdXJjZXMgb24gQ0hBQnMgYXJlIHVua25vd24uIEhlcmUsIHdlIHRyYWNrZWQgQ0hBQnMgYW5kIHBlcmZvcm1lZCBleHBlcmltZW50cyBpbiBmaXZlIGRpc3RpbmN0IGxha2VzIGFjcm9zcyB0aGUgTm9ydGhlcm4gVVMgaW5jbHVkaW5nIExha2UgRXJpZS4gRmVydGlsaXplcnMgd2l0aCBhbW1vbml1bSBhbmQgb3J0aG9waG9zcGhhdGUsIG1lbWJyYW5lICgwLjIgzrxtKS1maWx0ZXJlZCBzZXdhZ2UgKGRvbWluYXRlZCBieSByZWR1Y2VkIGZvcm1zIG9mIG5pdHJvZ2VuKSBzYW5kLWFuZCBtZW1icmFuZS1maWx0ZXJlZCBzZXdhZ2UgKGRvbWluYXRlZCBieSBuaXRyYXRlKSwgYW5kIGFuIGlub3JnYW5pYyBudXRyaWVudCBzb2x1dGlvbiBvZiBhbW1vbml1bSBhbmQgb3J0aG9waG9zcGhhdGUgd2VyZSB1c2VkIGFzIGV4cGVyaW1lbnRhbCBudXRyaWVudCBzb3VyY2VzIGZvciBDSEFCcyBhdCBOLWVxdWl2YWxlbnQsIGVudmlyb25tZW50YWxseSByZWFsaXN0aWMgY29uY2VudHJhdGlvbnMuIFBoeXRvcGxhbmt0b24gY29tbXVuaXRpZXMgd2VyZSBldmFsdWF0ZWQgZmx1b3JvbWV0cmljYWxseSwgbWljcm9zY29waWNhbGx5LCBhbmQgdmlhIGhpZ2ggdGhyb3VnaHB1dCBzZXF1ZW5jaW5nIG9mIHRoZSAxNlMgclJOQSBnZW5lLCBhbmQgbGV2ZWxzIG9mIG1pY3JvY3lzdGluIGFuZCB0aGUgzrQgMTUgTiBjb250ZW50IG9mIHBhcnRpY3VsYXRlIG9yZ2FuaWMgbml0cm9nZW4gKM60UE8gMTUgTikgd2VyZSBxdWFudGlmaWVkLiBGZXJ0aWxpemVyIGFuZCBib3RoIHNvdXJjZXMgb2Ygd2FzdGV3YXRlciBpbmNyZWFzZWQgdGhlIGFidW5kYW5jZSBvZiBjeWFub2JhY3RlcmlhIGluIGFsbCBleHBlcmltZW50cyBhY3Jvc3MgYWxsIGZpdmUgbGFrZXMgKCBwICZhbXA7bHQ7IDAuMDUgZm9yIGFsbCkgd2hlcmVhcyBlZmZlY3RzIG9uIGV1a2FyeW90aWMgcGh5dG9wbGFua3RvbiB3ZXJlIGxpbWl0ZWQuIFNhbmQtZmlsdGVyZWQgc2V3YWdlIGNvbnRhaW5lZCBsZXNzIFAsIG9yZ2FuaWMgbWF0dGVyLCBhbmQgYW1tb25pdW0gYnV0IG1vcmUgbml0cmF0ZSBhbmQgaGFkIGEgMjUlIGxlc3MgcG90ZW50IHN0aW11bGF0b3J5IGVmZmVjdCBvbiBjeWFub2JhY3RlcmlhIHRoYW4gbWVtYnJhbmUtZmlsdGVyZWQgc2V3YWdlLCBzdWdnZXN0aW5nIG5pdHJpZmljYXRpb24gbWF5IHBsYXkgYSByb2xlIGluIHJlZHVjaW5nIENIQUJzLiBGZXJ0aWxpemVyIGluY3JlYXNlZCBtaWNyb2N5c3RpbiBsZXZlbHMgYW5kIGRlY3JlYXNlZCB0aGUgzrRQTyAxNSBOIHdoZXJlYXMgd2FzdGV3YXRlciBpbmNyZWFzZWQgzrRQTyAxNSBOICggcCAmYW1wO2x0OyAwLjA1IGZvciBhbGwpLiBNaWNyb2N5c3RpcyB3YXMgdGhlIGdlbnVzIG1vc3QgY29uc2lzdGVudGx5IHByb21vdGVkIGJ5IG51dHJpZW50IHNvdXJjZXMgKCBwICZhbXA7bHQ7IDAuMDUgaW4gYWxsIGV4cGVyaW1lbnRzKSwgZm9sbG93ZWQgYnkgQ3lhbm9iaXVtICggcCAmYW1wO2x0OyAwLjA1IGluIDUwJSBvZiBleHBlcmltZW50cyksIHdpdGggaW5jcmVhc2VzIGluIE1pY3JvY3lzdGlzIGJpb21hc3MgY29uc2lzdGVudGx5IGVsaWNpdGVkIGJ5IG1lbWJyYW5lLWZpbHRlcmVkIHdhc3Rld2F0ZXIuIENvbGxlY3RpdmVseSwgcmVzdWx0cyBkZW1vbnN0cmF0ZSB0aGF0IGRpZmZlcmluZyB0eXBlcyBvZiBzZXdhZ2UgZGlzY2hhcmdlIGFuZCBmZXJ0aWxpemVycyBjYW4gcHJvbW90ZSBDSEFCIGludGVuc2l0eSBhbmQgdG94aWNpdHksIHdoaWxlIGNvbmN1cnJlbnRseSBhbHRlcmluZyBDSEFCIGRpdmVyc2l0eSBhbmQgzrRQTyAxNSBOLiBXaGlsZSBtZW1icmFuZS1maWx0ZXJlZCBzZXdhZ2UgY29uc2lzdGVudGx5IGZhdm9yZWQgTWljcm9jeXN0aXMgLCB0aGUgZGlzY2hhcmdlIG9mIHNld2FnZSB0aHJvdWdoIHNhbmRzIG11dGVkIGJsb29tIGludGVuc2l0eSBzdWdnZXN0aW5nIHNhbmQtYmVkcyBtYXkgcmVwcmVzZW50IGEgdG9vbCB0byByZW1vdmUga2V5IG51dHJpZW50cyBhbmQgcGFydGlhbGx5IG1pdGlnYXRlIENIQUJzLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzg5L2ZtaWNiLjIwMjQuMTQ2NDY4NiIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cuZnJvbnRpZXJzaW4ub3JnL2pvdXJuYWxzL21pY3JvYmlvbG9neS9hcnRpY2xlcy8xMC4zMzg5L2ZtaWNiLjIwMjQuMTQ2NDY4Ni9wZGYiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMS0wNiIsImFjY2Vzc2VkRGF0ZSI6IjIwMjYtMDIiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZyb250aWVycyBpbiBNaWNyb2Jpb2xvZ3kiLCJ2b2x1bWUiOjE1LCJpbXBhY3RGYWN0b3IiOiIzLjk5In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTQ2NDY4NiwibGFzdFBhZ2UiOjE0NjQ2ODZ9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRnJvbnRpZXJzIE1lZGlhIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo3MywiY2l0ZWRCeUNvdW50Ijo5fX0sInF1b3RlIjoiZGVyaXZlZCBudXRyaWVudHMgYWx0ZXIgdGhlIGludGVuc2l0eSwgZGl2ZXJzaXR5LCBhbmQgdG94aWNpdHkgb2YgaGFybWZ1bCBjeWFub2JhY3RlcmlhbCBibG9vbXMgLi4uIEN5YW5vYmFjdGVyaWFsIGhhcm1mdWwgYWxnYWwgYmxvb21zIChDSEFCcykgYXJlIHByb21vdGVkIGJ5IGV4Y2Vzc2l2ZSBudXRyaWVudCBsb2FkaW5nIGFuZCwgd2hpbGUgZmVydGlsaXplcnMgYW5kIHNld2FnZSBhcmUgdGhlIG1vc3QgcHJldmFsZW50IGV4dGVybmFsIG51dHJpZW50IHNvdXJjZXMgaW4gbW9zdCB3YXRlcnNoZWRzLCB0aGUgZGlmZmVyZW50aWFsIGVmZmVjdHMgb2YgdGhlc2UgbnV0cmllbnQgLi4uIG9yZ2FuaWMgbml0cm9nZW4gKM60UE8xNU4pIHdlcmUgcXVhbnRpZmllZC4gRmVydGlsaXplciBhbmQgYm90aCBzb3VyY2VzIG9mIHdhc3Rld2F0ZXIgaW5jcmVhc2VkIHRoZSBhYnVuZGFuY2Ugb2YgLi4uIHBoeXRvcGxhbmt0b24gd2VyZSBsaW1pdGVkLiBTYW5kLWZpbHRlcmVkIHNld2FnZSBjb250YWluZWQgbGVzcyBQLCBvcmdhbmljIG1hdHRlciwgYW5kIGFtbW9uaXVtIGJ1dCBtb3JlIG5pdHJhdGUgYW5kIC4uLiBkaWZmZXJpbmcgdHlwZXMgb2Ygc2V3YWdlIGRpc2NoYXJnZSBhbmQgZmVydGlsaXplcnMgY2FuIHByb21vdGUgQ0hBQiBpbnRlbnNpdHkgYW5kIHRveGljaXR5LCB3aGlsZSBjb25jdXJyZW50bHkgLi4uIHNld2FnZSB0aHJvdWdoIHNhbmRzIG11dGVkIGJsb29tIGludGVuc2l0eSBzdWdnZXN0aW5nIHNhbmQtYmVkcyBtYXkgcmVwcmVzZW50IGEgdG9vbCB0byByZW1vdmUga2V5IG51dHJpZW50cyBhbmQgLi4uICggV2F0c29uIGV0IGFsLiwgMjAxNiA7IEJ1bGxlcmphaG4gZXQgYWwuLCAyMDE2IDsgSHVpc21hbiBldCBhbC4sIDIwMTggKSBhbmQgZXhjZXNzaXZlIG51dHJpZW50IGxvYWRpbmcgaXMgdGhlIHByb2Nlc3MgbW9zdCBjb21tb25seSBjaXRlZCBhcyB0aGUgZW52aXJvbm1lbnRhbCBmYWN0b3IgcHJvbW90aW5nIENIQUJzICgifV0%3D. Pesticide residues from industrial washes, akin to agricultural inputs, selectively suppress zooplankton grazers like Daphnia, sparing resilient cyanobacteria and perpetuating blooms (Çelekli̇, 2024; Shahbaz et al., 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1kIiwiZmFtaWx5IjoiU2hhaGJheiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBaGJhciIsImZhbWlseSI6IkFsYW0ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTW9oYW1tYWQgTWFzcm9vciIsImZhbWlseSI6IlphZmFyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1vaGFtbWVkIEFhc2lmIiwiZmFtaWx5IjoiU3VsYWltYW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW51cG1hIiwiZmFtaWx5IjoiS3VtYXJpIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkcuIEsuIiwiZmFtaWx5IjoiU2hhcm1hIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNoYWhsYSIsImZhbWlseSI6Illhc21pbiJ9fV0sImlkcyI6eyJpc3NuIjpbIjEyMTUtMTYxMCIsIjIwNjEtOTU4OCJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMjQ3OC9vcmh1LTIwMjMtMDAxNiIsIm9wZW5hbGV4IjoiVzQzODk1MTAwMzcifSwidGl0bGUiOiJDb21wYXJpc29uIG9mIGF2aWFuIGRpdmVyc2l0eSBiZXR3ZWVuIG1hbmFnZWQgYW5kIHVubWFuYWdlZCB3ZXRsYW5kcyBpbiBQYXRuYSwgQmloYXIsIEluZGlhIiwiYWJzdHJhY3QiOiJBYnN0cmFjdCBXZSBjb21wYXJlZCB0aGUgYmlyZCBkaXZlcnNpdHkgb2YgYSBtYW5hZ2VkIChSYWpkaGFuaSBKYWxhc2hheSkgYW5kIGFuIHVubWFuYWdlZCB3ZXRsYW5kIChNdXN0YWZhcHVyKSBpbiBQYXRuYSwgQmloYXIsIEluZGlhLiBXZSByZWNvcmRlZCBiaXJkIHNwZWNpZXMgYnkgdXNpbmcgcG9pbnQgY291bnRzIGFuZCBmaXhlZC1yb3V0ZSBtb25pdG9yaW5nLiBBdCBSYWpkaGFuaSBKYWxhc2hheSwgYSB0b3RhbCBvZiA3MyBzcGVjaWVzIHdlcmUgcmVjb3JkZWQsIG9mIHdoaWNoIDE1IHNwZWNpZXMgd2VyZSB3aW50ZXIgdmlzaXRvcnMuIFR3byBzcGVjaWVzIHdlcmUgaW4gdGhlIE5lYXItdGhyZWF0ZW5lZCBjYXRlZ29yeTogRmVycnVnaW5vdXMgRHVjayAoQXl0aHlhIG55cm9jYSkgYW5kIEFsZXhhbmRyaW5lIFBhcmFrZWV0IChQc2l0dGFjdWxhIGV1cGF0cmlhKSAsIHdoaWxlIHRoZSBvdGhlcnMgd2VyZSBvZiBsZWFzdCBjb25jZXJuLiBBdCBNdXN0YWZhcHVyIHdldGxhbmQsIDY3IHNwZWNpZXMgd2VyZSByZWNvcmRlZCB3aXRoIDExIHNwZWNpZXMgYXMgd2ludGVyIHZpc2l0b3JzLiBTaW1wc29u4oCZcyBzcGVjaWVzIGRpdmVyc2l0eSBpbmRleCB3YXMgMC43OCBhdCBSYWpkaGFuaSBKYWxhc2hheSwgYW5kIDAuODEgYXQgTXVzdGFmYXB1ciB3ZXRsYW5kLiBUaGUgbnVtYmVyIG9mIHNwZWNpZXMgb2Ygd2FkZXJzIHdhcyAxNyBhdCBNdXN0YWZhcHVyIHdldGxhbmQgYW5kIDExIGF0IFJhamRoYW5pIEphbGFzaGF5LiBUaGUgdG90YWwgbnVtYmVyIG9mIExlc3NlciBXaGlzdGxpbmctZHVjayAoRGVuZHJvY3lnbmEgamF2YW5pY2EpICwgYSByZXNpZGVudCBiaXJkIHdhcyBhbG1vc3QgZG91YmxlICg1MjYpIGF0IE11c3RhZmFwdXIgd2V0bGFuZCB0aGFuIHRoYXQgYXQgUmFqZGhhbmkgSmFsYXNoYXkgKDIzNCkuIFRoZSBuaXRyYXRlIGxldmVsIHdhcyB0d2ljZSBoaWdoZXIgYW5kIHRoZSBwaG9zcGhhdGUgbGV2ZWwgd2FzIGZvdXIgdGltZXMgaGlnaGVyIGF0IE11c3RhZmFwdXIgd2V0bGFuZCB0aGFuIHRob3NlIGF0IFJhamRoYW5pIEphbGFzaGF5LiBUaGUgZGlmZmVyZW5jZSBpbiBzcGVjaWVzIG51bWJlciBhbmQgY29tcG9zaXRpb24gYmV0d2VlbiB0aGUgdHdvIHdldGxhbmRzIG1heSBiZSBhdHRyaWJ1dGVkIHRvIHRoZSBldXRyb3BoaWNhdGlvbiByZXN1bHRpbmcgZnJvbSBoaWdoIG5pdHJhdGUgYW5kIHBob3NwaGF0ZSBsZXZlbHMgYXQgTXVzdGFmYXB1ciB3ZXRsYW5kLiBVbmZvcnR1bmF0ZWx5LCB0aGUgTXVzdGFmYXB1ciB3ZXRsYW5kIGlzIHVuZGVyIHRocmVhdCBkdWUgdG8gaHVtYW4gaW5mbHVlbmNlcy4gVGhlIHByb3RlY3Rpb24gYW5kIHN1c3RhaW5hYmxlIG1hbmFnZW1lbnQgb2YgbmF0dXJhbCB3ZXRsYW5kcyBpcyByZXF1aXJlZCBmb3Igc2F2aW5nIHRoZSBiaW9kaXZlcnNpdHkgb2YgdGhlIGFyZWEuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjI0Nzgvb3JodS0yMDIzLTAwMTYiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vc2NpZW5kby5jb20vcGRmLzEwLjI0Nzgvb3JodS0yMDIzLTAwMTYiLCJpc3N1ZWREYXRlIjoiMjAyMy0xMi0wMSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6Ik9ybmlzIEh1bmdhcmljYSIsImlzc3VlIjoyLCJ2b2x1bWUiOjMxLCJpbXBhY3RGYWN0b3IiOiIwLjkyIn0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTMsImxhc3RQYWdlIjoyOH19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJIdW5nYXJpYW4gT3JuaXRob2xvZ2ljYWwgYW5kIE5hdHVyZSBDb25zZXJ2YXRpb24gU29jaWV0eSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MjcsImNpdGVkQnlDb3VudCI6MX19LCJxdW90ZSI6Ikhvd2V2ZXIsIGhpZ2ggbGV2ZWxzIG9mIGV1dHJvcGhpY2F0aW9uIG1heSBhbHNvIHRyaWdnZXIgdGhlIHByb2xpZmVyYXRpb24gb2YgZ3JlZW4gYWxnYWUgYW5kICBkZWNsaW5lIG9mIG1hY3JvcGh5dGVzIGxlYWRpbmcgdG8gYSBoaWdoLXR1cmJpZGl0eSBzdGF0ZSBvZiB3YXRlciwgaW4gd2hpY2ggZmV3IHNwZWNpZXMgY2FuICBzdXJ2aXZlIChTY2hlZmZlciBldCBhbC4gMTk5MykuIFRvIGFkZHJlc3MgdGhlIGV1dHJvcGhpY2F0aW9uIGFuZCBleGNlc3MgbnV0cmllbnQgaXNzdWVzIGluICBhcXVhdGljIGVjb3N5c3RlbXMsIHBoeXRvcmVtZWRpYXRpb24gb2ZmZXJzIGEgcHJvbWlzaW5nIHNvbHV0aW9uIChMaXUgZXQgYWwuIDIwMjEpLiBBcXVhdGljICBwbGFudHMgbGlrZSBkdWNrd2VlZHMsIHdhdGVyIGxldHR1Y2UgKFBpc3RpYSBzdHJhdGlvdGVzKSwgYW5kIHdhdGVyIGxpbGllcyBoYXZlIGRlbW9uc3RyYXRlZCAgdGhlaXIgYWJpbGl0eSB0byBjb21iYXQgZXV0cm9waGljYXRpb24gYnkgYWJzb3JiaW5nIGFuZCBhc3NpbWlsYXRpbmcgZXhjZXNzIHBob3NwaG9ydXMgYW5kICBuaXRyb2dlbiAoTmFoYXIgJiBIb3F1ZSAyMDIxKS4gSG93ZXZlciwgdGhlc2UgcGxhbnRzIGNhbiBzb21ldGltZXMgZ3JvdyB1bmNvbnRyb2xsYWJseSwgIHBvdGVudGlhbGx5IGNvdmVyaW5nIHRoZSB3YXRlciBzdXJmYWNlcy4gVG8gYXZvaWQgc3VjaCBzY2VuYXJpb3MsIHJlZ3VsYXIgbW9uaXRvcmluZyBpcyBlc3NlbnRpYWwgKExpdSBldCBhbC4gMjAyMSkuIFByb21vdGluZyBwdWJsaWMgYXdhcmVuZXNzIGlzIHRoZSBjb3JuZXJzdG9uZSBmb3IgbW9iaWxpemluZyBzdXBwb3J0IGFuZCBlbnN1cmluZyBlZmZlY3RpdmUgZXV0cm9waGljYXRpb24gbWFuYWdlbWVudC4gQ29udHJvbGxlZCBlcmFkaWNhdGlvbiBvZiBleGNlc3MgIGZsb3JhIHdoZW4gbmVjZXNzYXJ5IGlzIHZpdGFsIHRvIHByZXZlbnQgb3Zlcmdyb3d0aC4gVGhpcyBpbnRlZ3JhdGVkIGFwcHJvYWNoIGNhbiBlZmZlY3RpdmVseSAgcmVkdWNlIGV1dHJvcGhpY2F0aW9uIGluIHRoZSB3YXRlciBib2RpZXMgKFdhbmcgZXQgYWwuIDIwMjIpLiBFYWNoIHNwZWNpZXMgb2Ygd2V0bGFuZC1kZXBlbmRlbnQgYmlyZCBoYXMgYSB1bmlxdWUgYW5kIGNvbXBsZXggc2V0IG9mIG5lZWRzIGZvciB3ZXRsYW5kIGhhYml0YXRzIHRoYXQgbWFrZXMgaXQgZGlmZmljdWx0IHRvIGdlbmVyYWxpemUgYWJvdXQgaG93IGxvc3Mgb3IgZGVncmFkYXRpb24gb2Ygd2V0bGFuZHMgYWZmZWN0cyBiaXJkIHBvcHVsYXRpb25zLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo3OS4yOTU2MzkwMzgwODU5NCwibGVmdCI6MTAuMzAzMDA2MTcyMTgwMTc2LCJ3aWR0aCI6NzkuOTUyNzI4MjcxNDg0MzgsImhlaWdodCI6MS43MDkxOTc4Nzg4Mzc1ODU0fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjgxLjIxMjUzOTY3Mjg1MTU2LCJsZWZ0IjoxMC4zMDMwMDYxNzIxODAxNzYsIndpZHRoIjo3OS45NTkyODE5MjEzODY3MiwiaGVpZ2h0IjoxLjcwOTE5Nzg3ODgzNzU4NTR9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6ODMuMTI5NDQ3OTM3MDExNzIsImxlZnQiOjEwLjMwMzAwNjE3MjE4MDE3Niwid2lkdGgiOjc5Ljk0MjcyNjEzNTI1MzksImhlaWdodCI6MS43MDkxOTc4Nzg4Mzc1ODU0fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjg1LjA0NjM0ODU3MTc3NzM0LCJsZWZ0IjoxMC4zMDMwMDgwNzk1Mjg4MDksIndpZHRoIjo3OS45Mzk0NTMxMjUsImhlaWdodCI6MS43MDkxOTc4Nzg4Mzc1ODU0fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjg2Ljk2MzI1NjgzNTkzNzUsImxlZnQiOjEwLjMwMzAxMDk0MDU1MTc1OCwid2lkdGgiOjc5Ljk0MTg0ODc1NDg4MjgxLCJoZWlnaHQiOjEuNzA5MTk3ODc4ODM3NTg1NH19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo4OC44ODAxNjUxMDAwOTc2NiwibGVmdCI6MTAuMzAzMDEzODAxNTc0NzA3LCJ3aWR0aCI6NzkuOTM3NSwiaGVpZ2h0IjoxLjcwOTE5Nzg3ODgzNzU4NTR9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6OTAuNzk3MDY1NzM0ODYzMjgsImxlZnQiOjEwLjMwMzAxMzgwMTU3NDcwNywid2lkdGgiOjc5LjkzOTY1MTQ4OTI1NzgxLCJoZWlnaHQiOjEuNzA5MTk3ODc4ODM3NTg1NH19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo5Mi43MTM5NzM5OTkwMjM0NCwibGVmdCI6MTAuMzAzMDEzODAxNTc0NzA3LCJ3aWR0aCI6NzkuOTUwMzI1MDEyMjA3MDMsImhlaWdodCI6MS43MDkxOTc4Nzg4Mzc1ODU0fX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjEyLjIwNzk5MDY0NjM2MjMwNSwibGVmdCI6MTAuMzAzMDI0MjkxOTkyMTg4LCJ3aWR0aCI6NzkuOTUyNTE0NjQ4NDM3NSwiaGVpZ2h0Ijo3LjQ1OTkyMjc5MDUyNzM0NH19LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjoxOS44NzU2MjU2MTAzNTE1NjIsImxlZnQiOjEyLjEyMTMyNDUzOTE4NDU3LCJ3aWR0aCI6NzguMTE5MjI0NTQ4MzM5ODQsImhlaWdodCI6MS43MDkxOTc4Nzg4Mzc1ODU0fX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjIxLjc5MjUzMzg3NDUxMTcyLCJsZWZ0IjoxMC4zMDMwMjkwNjAzNjM3Nywid2lkdGgiOjc5Ljk0OTI0OTI2NzU3ODEyLCJoZWlnaHQiOjEuNzA5MTk3ODc4ODM3NTg1NH19LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjoyMy43MDk0MzY0MTY2MjU5NzcsImxlZnQiOjEwLjMwMzAyOTA2MDM2Mzc3LCJ3aWR0aCI6NzkuOTUyMjcwNTA3ODEyNSwiaGVpZ2h0IjoxLjcwOTE5Nzg3ODgzNzU4NTR9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIyNSJ9fSx7Im9yaWdpbiI6NCwid29yayI6eyJ0eXBlIjo4LCJzdWJ0eXBlIjp7Im9uZW9mS2luZCI6ImFydGljbGVUeXBlIiwiYXJ0aWNsZVR5cGUiOjF9LCJjb250cmlidXRvcnMiOlt7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBYnV6ZXIiLCJmYW1pbHkiOiLDh2VsZWtsacyHIn19XSwiaWRzIjp7Imlzc24iOltdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMjEyMDMvcnMuMy5ycy01MDc4MTc2L3YxIiwib3BlbmFsZXgiOiJXNDQwNDUyOTQxOCJ9LCJ0aXRsZSI6IlRoZSBmaXJzdCBlY28tYXNzZXNzbWVudCBvZiBwaHl0b3BsYW5rdG9uIGFzc2VtYmxhZ2VzIGFzc29jaWF0ZWQgd2l0aCBlbnZpcm9ubWVudGFsIGNvbmRpdGlvbnMgb2YgZGlmZmVyZW50IGxha2VzIGluIHRoZSBXZXN0ZXJuIEJsYWNrIFNlYSBiYXNpbiBvZiBUw7xya2l5ZSIsImFic3RyYWN0IjoiPHRpdGxlPkFic3RyYWN0PC90aXRsZT4gTGFrZXMgcGxheSBhIHZpdGFsIHJvbGUgaW4gc3VwcG9ydGluZyBiaW9kaXZlcnNpdHksIHByb3ZpZGluZyB3YXRlciByZXNvdXJjZXMsIHJlZ3VsYXRpbmcgY2xpbWF0ZSwgY3ljbGluZyBudXRyaWVudHMsIGFuZCBvZmZlcmluZyByZWNyZWF0aW9uYWwgb3Bwb3J0dW5pdGllcy4gRGVzcGl0ZSB0aGVpciBpbXBvcnRhbmNlIGZvciBlbnZpcm9ubWVudGFsIGhlYWx0aCBhbmQgaHVtYW4gd2VsbC1iZWluZywgbGFrZXMgZmFjZSBzaWduaWZpY2FudCBwcmVzc3VyZXMgaW4gdGhlIEFudGhyb3BvY2VuZSBlcmEuIFRoZSBwcmVzZW50IHdvcmsgc2Vla3MgdG8gYXNzZXNzIHRoZSBzcGVjaWVzLWVudmlyb25tZW50IGludGVyYWN0aW9ucyBhbmQgdGhlIGVjb2xvZ2ljYWwgc3RhdHVzIG9mIHNpeCBsYWtlcyBpbiB0aGUgV2VzdGVybiBCbGFjayBTZWEgYmFzaW4gb2YgVMO8cmtpeWUgdXRpbGl6aW5nIHBoeXRvcGxhbmt0b24gbWV0cmljcyBkdXJpbmcgd2V0IGFuZCBkcnkgcGVyaW9kcy4gQ2Fub25pY2FsIGNvcnJlc3BvbmRlbmNlIGFuYWx5c2lzIHJldmVhbGVkIGEgc2lnbmlmaWNhbnQgY29ycmVsYXRpb24gZXF1YWwgdG8gOTguNSUgYmV0d2VlbiBwaHl0b3BsYW5rdG9uIHNwZWNpZXMgYW5kIGVudmlyb25tZW50YWwgc3RyZXNzb3JzIGR1cmluZyB0d28gaHlkcm9sb2dpY2FsIHBlcmlvZHMuIEVsZWN0cmljYWwgY29uZHVjdGl2aXR5IChFQyksIHBILCB0b3RhbCBvcmdhbmljIGNhcmJvbiAoVE9DKSwgYW5kIHRlbXBlcmF0dXJlIHdlcmUgdGhlIG1vc3QgaW5mbHVlbnRpYWwgZW52aXJvbm1lbnRhbCBmYWN0b3JzIGFmZmVjdGluZyBwaHl0b3BsYW5rdG9uIGRpc3RyaWJ1dGlvbiBpbiBsYWtlcyAocD0wLjAwMikuIExha2UgU2FyxLFrdW0sIGEgYnJhY2tpc2ggaGFiaXRhdCwgd2FzIHVuZGVyIGhpZ2ggRUMgYW5kIHBIIHByZXNzdXJlIGFuZCBpcyBjaGFyYWN0ZXJpemVkIGJ5IHBvbGx1dGlvbi10b2xlcmFudCBzcGVjaWVzLiBMYWtlIFllbmnDp2HEn2EgYXNzb2NpYXRlZCB3aXRoIFRPQyBpcyBjaGFyYWN0ZXJpemVkIGJ5IHNvbWUgc3BlY2llcywgc3VjaCBhcyA8aXRhbGljPkFuYWJhZW5vcHNpczwvaXRhbGljPiA8aXRhbGljPm1pbGxlcmk8L2l0YWxpYz4sIDxpdGFsaWM%2BQ2hyb29jb2NjdXM8L2l0YWxpYz4gPGl0YWxpYz50dXJnaWR1czwvaXRhbGljPiw8aXRhbGljPiBQc2V1ZG9zY2hyb2VkZXJpYTwvaXRhbGljPiA8aXRhbGljPnJvYnVzdGE8L2l0YWxpYz4sIDxpdGFsaWM%2BQXBoYW5vY2Fwc2E8L2l0YWxpYz4gc3AuLCA8aXRhbGljPk1lcmlzbW9wZWRpYTwvaXRhbGljPiA8aXRhbGljPmdsYXVjYTwvaXRhbGljPiwgPGl0YWxpYz5NaWNyYWN0aW5pdW08L2l0YWxpYz4gPGl0YWxpYz5xdWFkcmlzZXR1bTwvaXRhbGljPiwgYW5kPGl0YWxpYz4gTWljcm9jeXN0aXM8L2l0YWxpYz4gPGl0YWxpYz5hZXJ1Z2lub3NhPC9pdGFsaWM%2BLiBMYWtlIEFiYW50IGlzIGxvY2F0ZWQgb24gdGhlIG9wcG9zaXRlIHNpZGUgb2YgRUMsIFRPQywgYW5kIHRlbXBlcmF0dXJlLCB3aGljaCB3YXMgYXNzb2NpYXRlZCB3aXRoIHNvbWUgc3BlY2llcyBzdWNoIGFzIDxpdGFsaWM%2BQ3ltYmVsbGEgYWZmaW5pczwvaXRhbGljPiwgPGl0YWxpYz5BY2huYW50aGlkaXVtPC9pdGFsaWM%2BIDxpdGFsaWM%2BbWludXRpc3NpbXVtPC9pdGFsaWM%2BLCA8aXRhbGljPkVuY3lvbmVtYTwvaXRhbGljPiA8aXRhbGljPm1pbnV0dW08L2l0YWxpYz4sIDxpdGFsaWM%2BRS48L2l0YWxpYz4gPGl0YWxpYz5zaWxlc2lhY3VtPC9pdGFsaWM%2BLCBhbmQ8aXRhbGljPiBEaW5vYnJ5b248L2l0YWxpYz4gPGl0YWxpYz5kaXZlcmdlbnM8L2l0YWxpYz4uIFJlc3VsdHMgb2YgdGhlIHBoeXRvLWFzc2Vzc21lbnQgZGlzcGxheWVkIHRoYXQgdGhlIGVjb2xvZ2ljYWwgc3RhdHVzIChFUykgb2YgdGhlIHNhbXBsaW5nIHN0YXRpb25zIGR1cmluZyB0aGUgcmFpbnkgYW5kIGRyeSBwZXJpb2RzIHZhcmllZCBmcm9tIGJhZCB0byBoaWdoLiBUaGUgbW9kaWZpZWQgcGh5dG9wbGFua3RvbiB0cm9waGljIGluZGV4IChNUFRJKSBleGhpYml0ZWQgdGhhdCBhIG1vZGVyYXRlIEVTIHdhcyBmb3VuZCBpbiBMYWtlcyBTYXLEsWt1bSBhbmQgWWVuacOnYcSfYSwgd2hpbGUgb3RoZXJzIGhhZCBhIGdvb2QgRVMuIFRoZSBwcmVzZW50IHN0dWR5IGNvbu%2BsgXJtZWQgdGhhdCBwaHl0b3BsYW5rdG9uIGNvbW11bml0aWVzIGFyZSBzdHJvbmdseSBsaW5rZWQgdG8gdGhlIGVjb2xvZ2ljYWwgc3RhdHVzIG9mIGxha2VzIGluIHRoZSBXZXN0ZXJuIEJsYWNrIFNlYSBiYXNpbiwgd2hpY2ggY291bGQgYmUgYXNzZXNzZWQgdXNpbmcgdGhlIE1QVEkuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjIxMjAzL3JzLjMucnMtNTA3ODE3Ni92MSIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cucmVzZWFyY2hzcXVhcmUuY29tL2FydGljbGUvcnMtNTA3ODE3Ni9sYXRlc3QucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjQtMTEtMTkiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA4IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJSZXNlYXJjaCBTcXVhcmUgKFJlc2VhcmNoIFNxdWFyZSkifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiUmVzZWFyY2ggU3F1YXJlIChVbml0ZWQgU3RhdGVzKSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NTIsImNpdGVkQnlDb3VudCI6MH19LCJxdW90ZSI6IkRlc3BpdGUgdGhlaXIgZWNvbG9naWNhbCBzaWduae%2B%2FvWNhbmNlLCB0aGVzZSB3YXRlciBib2RpZXMgYXJlIGluY3JlYXNpbmdseSB0aHJlYXRlbmVkIGJ5IGFudGhyb3BvZ2VuaWMgYWN0aXZpdGllcyAoZS5nLiwgdXJiYW5pemF0aW9uLCBhZ3JpY3VsdHVyYWwgbGFuZCB1c2UsIHdhc3Rld2F0ZXIgZGlzY2hhcmdlLCByZWVkIGN1dHRpbmcsIHNhbmQgcmVtb3ZhbCwgZXRjLikgYW5kIGNsaW1hdGUgY2hhbmdlIChIw6RkZXIgZXQgYWwuIDIwMjA7IMOHZWxla2xpIGV0IGFsLiAyMDI0KS4gTnV0cmllbnQgZW5yaWNobWVudCBpbiBsZW50aWMgZWNvc3lzdGVtcyBpbXBhY3RzIG5vdCBvbmx5IHRoZSBwaHlzaWNvY2hlbWljYWwgZmFjdG9ycyBvZiBsYWtlcyBidXQgYWxzbyBuZWdhdGl2ZWx5IGFmZmVjdHMgYXF1YXRpYyBsaWZlLCBjYXVzaW5nIGhhcm1mdWwgYWxnYWwgYmxvb21zLCBleGNlc3NpdmUgZ3Jvd3RoIG9mIG51aXNhbmNlIG1hY3JvcGh5dGVzLCBhbmQgaW5jcmVhc2VkIO%2B%2FvXNoIG1vcnRhbGl0eSwgZXNwZWNpYWxseSBzZW5zaXRpdmUgc3BlY2llcyAoU3VyZXNoIGV0IGFsLiAyMDIzKS4gSXQgYWxzbyBkaXNydXB0cyB0aGUgaHlkcm9tb3JwaG9sb2d5IGFuZCBvdmVyYWxsIGZ1bmN0aW9uaW5nIG9mIHRoZXNlIGVjb3N5c3RlbXMuIERlc3BpdGUgZ2xvYmFsIHdhdGVyIHNob3J0YWdlcywgdGhlIGdyb3dpbmcgZGVncmFkYXRpb24gb2YgZWNvc3lzdGVtcyBhbmQgdGhlIG92ZXJ1c2Ugb2YgZnJlc2h3YXRlciByZXNvdXJjZXMgaGF2ZSBzaGlmdGVkIGF0dGVudGlvbiBiYWNrIHRvIGZyZXNod2F0ZXIgZWNvc3lzdGVtcyB0byBiZXR0ZXIgbWFuYWdlIGFuZCBwcmVzZXJ2ZSB3YXRlciByZXNvdXJjZXMgZ2xvYmFsbHkgKMOHZWxla2xpIGV0IGFsLiAyMDIxOyBTdXJlc2ggZXQgYWwuIDIwMjMpLiBBc3Nlc3NpbmcgdGhlIGVjb2xvZ2ljYWwgc3RhdHVzIG9mIGxha2VzIHRocm91Z2ggcGh5dG9wbGFua3RvbiBtZXRyaWNzIGlzIGEgY3J1Y2lhbCBjb21wb25lbnQgb2YgZW52aXJvbm1lbnRhbCBtb25pdG9yaW5nIGFuZCBtYW5hZ2VtZW50LCBhcyBpdCBhbGlnbnMgd2l0aCB0aGUgZ3VpZGVsaW5lcyBzZXQgYnkgdGhlIFdhdGVyIEZyYW1ld29yayBEaXJlY3RpdmUgKERpcmVjdGl2ZSAyMDAwKS4gUGh5dG9wbGFua3RvbiwgYXMgcHJpbWFyeSBwcm9kdWNlcnMsIGFyZSBzZW5zaXRpdmUgaW5kaWNhdG9ycyBvZiBjaGFuZ2VzIGluIHdhdGVyIHF1YWxpdHkgYW5kIGVjb3N5c3RlbSBoZWFsdGggKFJleW5vbGRzIGV0IGFsLiAyMDAyOyDDh2VsZWtsaSBldCBhbC4gMjAyM2EpLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjY2Ljk2NDIyNTc2OTA0Mjk3LCJsZWZ0Ijo0LjUzNDMxMzY3ODc0MTQ1NSwid2lkdGgiOjkwLjYxNDc2MTM1MjUzOTA2LCJoZWlnaHQiOjEuNzc1NTI5NjIzMDMxNjE2Mn19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjY5LjIzNjkwNzk1ODk4NDM4LCJsZWZ0Ijo0LjUzNDMxMzY3ODc0MTQ1NSwid2lkdGgiOjkwLjAxMDMwNzMxMjAxMTcyLCJoZWlnaHQiOjEuNzc1NTI5NjIzMDMxNjE2Mn19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjcxLjUwOTU5MDE0ODkyNTc4LCJsZWZ0Ijo0LjUzNDMxMzY3ODc0MTQ1NSwid2lkdGgiOjg1LjgxMjE3OTU2NTQyOTY5LCJoZWlnaHQiOjEuNzc1NTI5NjIzMDMxNjE2Mn19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjczLjc4MjI3MjMzODg2NzE5LCJsZWZ0Ijo0LjUzNDMxMzY3ODc0MTQ1NSwid2lkdGgiOjg2LjA3MDMxMjUsImhlaWdodCI6MS43NzU1Mjk2MjMwMzE2MTYyfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NzYuMDU0OTU0NTI4ODA4NiwibGVmdCI6NC41MzQzMTM2Nzg3NDE0NTUsIndpZHRoIjo4OS42MjM2NjQ4NTU5NTcwMywiaGVpZ2h0IjoxLjc3NTUyOTYyMzAzMTYxNjJ9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo3OC4zMjc2MjkwODkzNTU0NywibGVmdCI6NC41MzQzMTM2Nzg3NDE0NTUsIndpZHRoIjo4OS44NTk3NTY0Njk3MjY1NiwiaGVpZ2h0IjoxLjc3NTUyOTYyMzAzMTYxNjJ9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo4MC42MDAzMTg5MDg2OTE0LCJsZWZ0Ijo0LjUzNDMxMzY3ODc0MTQ1NSwid2lkdGgiOjg5LjY0MDgzODYyMzA0Njg4LCJoZWlnaHQiOjEuNzc1NTI5NjIzMDMxNjE2Mn19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjgyLjg3Mjk5MzQ2OTIzODI4LCJsZWZ0Ijo0LjUzNDMxMzY3ODc0MTQ1NSwid2lkdGgiOjkwLjgzNjYzOTQwNDI5Njg4LCJoZWlnaHQiOjQuMDQ4MTk0ODg1MjUzOTA2fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6ODcuNDE4MzQyNTkwMzMyMDMsImxlZnQiOjQuNTM0MzEzNjc4NzQxNDU1LCJ3aWR0aCI6MzMuNzkwOTM1NTE2MzU3NDIsImhlaWdodCI6MS43NzU1Mjk2MjMwMzE2MTYyfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6OTEuMjA2MTQ2MjQwMjM0MzgsImxlZnQiOjQuNTM0MzEzNjc4NzQxNDU1LCJ3aWR0aCI6ODQuNjc1Mzc2ODkyMDg5ODQsImhlaWdodCI6MS43NzU1Mjk2MjMwMzE2MTYyfX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6OTMuNDc4ODIwODAwNzgxMjUsImxlZnQiOjQuNTM0MzEzNjc4NzQxNDU1LCJ3aWR0aCI6ODkuOTgzODQ4NTcxNzc3MzQsImhlaWdodCI6MS43NzU1Mjk2MjMwMzE2MTYyfX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6NC43NDk2NDM4MDI2NDI4MjIsImxlZnQiOjQuNTM0MzEzNjc4NzQxNDU1LCJ3aWR0aCI6ODcuOTU3ODYyODU0MDAzOSwiaGVpZ2h0IjoxLjc3NTUzMDU3NjcwNTkzMjZ9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo3LjAyMjMwOTMwMzI4MzY5MSwibGVmdCI6NC41MzQzMTM2Nzg3NDE0NTUsIndpZHRoIjo5MC42NjY1MTE1MzU2NDQ1MywiaGVpZ2h0IjoxLjc3NTUzMDU3NjcwNTkzMjZ9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIyIn19XQ%3D%3D. Internal feedbacks intensify: decaying algal biomass under low dissolved oxygen (DO <2 mg/L) mobilizes sedimentary phosphorus via anoxic mineralization, sustaining eutrophication despite variable external inputs (Haxthausen, 2021; Paerl, 2018; Swann et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhhbnMgVy4iLCJmYW1pbHkiOiJQYWVybCJ9fV0sImlkcyI6eyJpc3NuIjpbXSwiaXNibiI6W10sImRvaSI6IjEwLjE3NjE1L3luNTUtYjQzOCIsIm9wZW5hbGV4IjoiVzQzMDcwNjM3MzkifSwidGl0bGUiOiJNaXRpZ2F0aW5nIHRveGljIHBsYW5rdG9uaWMgY3lhbm9iYWN0ZXJpYWwgYmxvb21zIGluIGFxdWF0aWMgZWNvc3lzdGVtcyBmYWNpbmcgaW5jcmVhc2luZyBhbnRocm9wb2dlbmljIGFuZCBjbGltYXRpYyBwcmVzc3VyZXMiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTc2MTUveW41NS1iNDM4IiwicmVzb3VyY2VVcmwiOiJodHRwczovL2Nkci5saWIudW5jLmVkdS9kb3dubG9hZHMvd2gyNDcyOTBnIiwiaXNzdWVkRGF0ZSI6IjIwMTgtMDEtMDEiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA2IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJDYXJvbGluYSBEaWdpdGFsIFJlcG9zaXRvcnkgKFVuaXZlcnNpdHkgb2YgTm9ydGggQ2Fyb2xpbmEgYXQgQ2hhcGVsIEhpbGwpIiwiaW1wYWN0RmFjdG9yIjoiMC4xMCJ9LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJVbml2ZXJzaXR5IG9mIE5vcnRoIENhcm9saW5hIGF0IENoYXBlbCBIaWxsIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjowLCJjaXRlZEJ5Q291bnQiOjB9fSwicXVvdGUiOiJUaGlzIGxlYWRzIHRvIGNvbnRpbnVpbmcgZGVtYW5kIGZvciBOIHN1cHBsaWVzIHRvIHN1c3RhaW4gcHJpbWFyeSBwcm9kdWN0aW9uIGFuZCBwcm9tb3RlIGV1dHJvcGhpY2F0aW9uIFszOCwzOV0sICgyKSBhIGxlbmd0aHkgbGVnYWN5IG9mIFAgbG9hZGluZyB0aGF0IGlzIHN0b3JlZCBpbiBzZWRpbWVudHMsIHdoaWNoIGlzIHJlYWRpbHkgcmVjeWNsZWQgYW5kIHN1cHBvcnRzIGV1dHJvcGhpY2F0aW9uLCAoMykgTjIgZml4YXRpb24sIHdoaWxlIHByb3ZpZGluZyDigJxuZXfigJ0gTiwgZmFpbHMgdG8gbWVldCBlY29zeXN0ZW0tc2NhbGUgTiByZXF1aXJlbWVudHMgWzQwLDQxXS4gRnJvbSBhIG51dHJpZW50IG1hbmFnZW1lbnQgcGVyc3BlY3RpdmUsIHRoaXMgbWVhbnMgdGhhdCBjb250aW51ZWQgZXh0ZXJuYWwgbG9hZGluZyBvZiBOIHBsYXlzIGEgY3JpdGljYWwgcm9sZSBpbiBhY2NlbGVyYXRpbmcgZXV0cm9waGljYXRpb24gYW5kIHByb21vdGluZyBhbGdhbCBibG9vbXMgWzM2XS4gT25lIHRyb3VibGluZyBpbmRpY2F0b3Igb2YgTi1kcml2ZW4gZXV0cm9waGljYXRpb24gYW5kIEN5YW5vSEFCcyBpbiBmcmVzaHdhdGVyIHN5c3RlbXMgaXMgdGhlIGdlb2dyYXBoaWMgZXhwYW5zaW9uIGFuZCBpbmNyZWFzZSBmcmVxdWVuY3kgb2YgdGhlIG5vbi1OMiBmaXhpbmcgdG94aW4tcHJvZHVjaW5nIGdlbnVzIE1pY3JvY3lzdGlzIFs0Ml0sIGVzcGVjaWFsbHkgaW4gbGFyZ2UgbGFrZXMgKGUuZy4sIExha2UgRXJpZSwgQ2FuYWRhLVVTQSwgTGFrZSBLYXN1bWFndXJhLCBKYXBhbiwgTGFrZSBPa2VlY2hvYmVlLCBGbG9yaWRhLCBVU0EsIExha2UgVGFpaHUsIENoaW5hKS4gVGhpcyBnZW51cywgYXMgd2VsbCBhcyBhbm90aGVyIHBsYW5rdG9uaWMgbm9uLU4yIGZpeGluZyBDeWFub0hBQiBQbGFua3RvdGhyaXgsIGFuZCBzZXZlcmFsIGJlbnRoaWMgYW5hbG9ndWUgYmxvb20gdGF4YSAoZS5nLiwgbm9uLU4yIGZpeGluZyBMeW5nYnlhIGFuZCBPc2NpbGxhdG9yaWEgc3BwLiksIHNlcnZlIGFzIGluZGljYXRvcnMgb2YgTi1vdmVyLWVucmljaG1lbnQgWzM2LDQyXS4gVGhlIHByb2xpZmVyYXRpb24gb2YgdGhlc2Ugbm9uLU4yIGZpeGluZyBDeWFub0hBQnMgaXMgYW4gYWRkaXRpb25hbCBpbmRpY2F0b3IgdGhlIE4gaW5wdXQgcmVkdWN0aW9ucyBhcmUgbmVlZGVkIHRvIHJldmVyc2UgdGhpcyB0cmVuZC4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo3Ny40NDYyNTA5MTU1MjczNCwibGVmdCI6MTYuNDI4NTE0NDgwNTkwODIsIndpZHRoIjo3MC43MTQ3MjE2Nzk2ODc1LCJoZWlnaHQiOjEuNDUwNzk2NDg0OTQ3MjA0Nn19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjc5LjA1MDk3OTYxNDI1NzgxLCJsZWZ0IjoxMi44NTcwNjEzODYxMDgzOTgsIndpZHRoIjo3NC40OTQ5ODc0ODc3OTI5NywiaGVpZ2h0IjoxLjQ1MDc5NjQ4NDk0NzIwNDZ9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo4MC42NTU1ODYyNDI2NzU3OCwibGVmdCI6MTIuODAxOTYxODk4ODAzNzExLCJ3aWR0aCI6NzQuMzQwMDE5MjI2MDc0MjIsImhlaWdodCI6MS40NTA3OTY0ODQ5NDcyMDQ2fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6ODIuMjYwMTg1MjQxNjk5MjIsImxlZnQiOjEyLjg1NzA2MTM4NjEwODM5OCwid2lkdGgiOjc0LjI4NTE4Njc2NzU3ODEyLCJoZWlnaHQiOjEuNjY2OTc3NjQzOTY2Njc0OH19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjgzLjg2NDkwNjMxMTAzNTE2LCJsZWZ0IjoxMi44NTcwNjEzODYxMDgzOTgsIndpZHRoIjo3NC4yODUwMTg5MjA4OTg0NCwiaGVpZ2h0IjoxLjQ1MDc5NjQ4NDk0NzIwNDZ9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo4NS40Njk1MTI5Mzk0NTMxMiwibGVmdCI6MTIuODU3MDYxMzg2MTA4Mzk4LCJ3aWR0aCI6NzEuMTA0OTg4MDk4MTQ0NTMsImhlaWdodCI6MS40NTA3OTY0ODQ5NDcyMDQ2fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6ODcuMDc0MjM0MDA4Nzg5MDYsImxlZnQiOjE2LjQyODUxNDQ4MDU5MDgyLCJ3aWR0aCI6NzAuNzE0MTcyMzYzMjgxMjUsImhlaWdodCI6MS40NTA3OTY0ODQ5NDcyMDQ2fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6ODguNjc4ODQwNjM3MjA3MDMsImxlZnQiOjEyLjg1NzA2MTM4NjEwODM5OCwid2lkdGgiOjc0LjI4NjAyNjAwMDk3NjU2LCJoZWlnaHQiOjEuNjY2OTc3NjQzOTY2Njc0OH19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjkwLjI4MzQzOTYzNjIzMDQ3LCJsZWZ0IjoxMi44NTcwNjEzODYxMDgzOTgsIndpZHRoIjo3NC4yODU0MTU2NDk0MTQwNiwiaGVpZ2h0IjoxLjQ3NjgzMDQ4MjQ4MjkxMDJ9fSx7InBhZ2VOdW1iZXIiOjYsImJib3giOnsidG9wIjoxMC41MDY1MTM1OTU1ODEwNTUsImxlZnQiOjEyLjg1NzA2MTM4NjEwODM5OCwid2lkdGgiOjc0LjIwMjAxODczNzc5Mjk3LCJoZWlnaHQiOjEuNTg2NjA4NDA5ODgxNTkxOH19LHsicGFnZU51bWJlciI6NiwiYmJveCI6eyJ0b3AiOjEyLjExMTIzNzUyNTkzOTk0MSwibGVmdCI6MTIuODU3MDYxMzg2MTA4Mzk4LCJ3aWR0aCI6NzQuMjg1Njc1MDQ4ODI4MTIsImhlaWdodCI6MS42NjY5ODEzMzk0NTQ2NTA5fX0seyJwYWdlTnVtYmVyIjo2LCJiYm94Ijp7InRvcCI6MTMuNzE1ODM3NDc4NjM3Njk1LCJsZWZ0IjoxMi44NTcwNjEzODYxMDgzOTgsIndpZHRoIjo3NC4yODU5NzI1OTUyMTQ4NCwiaGVpZ2h0IjoxLjQ3NjgzNDE3Nzk3MDg4NjJ9fSx7InBhZ2VOdW1iZXIiOjYsImJib3giOnsidG9wIjoxNS4zMjA1NjE0MDg5OTY1ODIsImxlZnQiOjEyLjg1NzA2MTM4NjEwODM5OCwid2lkdGgiOjc0LjI4NTEwMjg0NDIzODI4LCJoZWlnaHQiOjEuNjY2ODY1MzQ4ODE1OTE4fX0seyJwYWdlTnVtYmVyIjo2LCJiYm94Ijp7InRvcCI6MTYuOTI1MTYzMjY5MDQyOTcsImxlZnQiOjEyLjg1NzA2MTM4NjEwODM5OCwid2lkdGgiOjc0LjI4NTM1NDYxNDI1NzgxLCJoZWlnaHQiOjEuNjY2OTgxMzM5NDU0NjUwOX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjYifX0seyJvcmlnaW4iOjQsIndvcmsiOnsidHlwZSI6OCwic3VidHlwZSI6eyJvbmVvZktpbmQiOiJhcnRpY2xlVHlwZSIsImFydGljbGVUeXBlIjoxfSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiS2FybCBSYXNtdXMgQXVndXN0IiwiZmFtaWx5IjoiSGF4dGhhdXNlbiJ9fV0sImlkcyI6eyJpc3NuIjpbXSwiaXNibiI6W10sIm1hZyI6IjMxNzcxMTMyMDciLCJvcGVuYWxleCI6IlczMTc3MTEzMjA3In0sInRpdGxlIjoiU2VkaW1lbnQgbWljcm9iaWFsIGZ1ZWwgY2VsbHMgZm9yIGxha2UgcmVzdG9yYXRpb24iLCJhYnN0cmFjdCI6IkV1dHJvcGhpY2F0aW9uIG9mIGxha2VzIGlzIHViaXF1aXRvdXMsIGFzIDMwLTQwICUgb2YgdGhlIHdvcmxk4oCZcyBsYWtlcyBhcmUgZXV0cm9waGljLCBhbmQgY29uZGl0aW9ucyBjb250aW51b3VzbHkgZGV0ZXJpb3JhdGluZy4gRXV0cm9waGljYXRpb24gaXMgYSBjb25kaXRpb24gb2Ygb3ZlcmZlcnRpbGl6YXRpb24sIHdoZXJlIGFsbCByZXF1aXJlbWVudHMgZm9yIHJhcGlkIGdyb3d0aCBvZiBhbGdhZSBhcmUgbWV0LCByZXN1bHRpbmcgaW4gYWxnYWwgYmxvb21zLiBUaGlzIGNhdXNlcyBhIHNlbGYtc3VzdGFpbmVkLCB2aWNpb3VzIGN5Y2xlIHdoZXJlIHBvb3IgZWNvbG9naWNhbCBjb25kaXRpb25zIGFyZSBtYWludGFpbmVkIHVudGlsIG51dHJpZW50cyBhcmUgbG9zdCBmcm9tIHRoZSBsYWtlIG92ZXIgbWFueSB5ZWFycy4gTGFrZSByZXN0b3JhdGlvbiBzZWVrcyB0byBicmVhayB0aGlzIHZpY2lvdXMgY3ljbGUuIEluYWRlcXVhdGUgYXZhaWxhYmlsaXR5IG9mIGFueSBzaW5nbGUgbnV0cmllbnQgaW4gd2lsbCBsaW1pdCBhbGdhbCBncm93dGgsIHR5cGljYWxseSBwaG9zcGhvcnVzIGlzIHRhcmdldGVkLiBUaGUgc2NvcGUgb2YgdGhpcyBQaEQgdGhlc2lzIHdhcyB0byBkZXZlbG9wLCB0ZXN0IGFuZCBvcHRpbWl6ZSBzZWRpbWVudCBtaWNyb2JpYWwgZnVlbCBjZWxscyAoU01GQykgYXMgYSBub3ZlbCBtZXRob2QgZm9yIGxha2UgcmVzdG9yYXRpb24uIFRoZSB0aGVzaXMgc3BhbnMgdGhlIHR3byBtYWluIHRvcGljcyBvZiBTTUZDcyBhbmQgbGFrZSByZXN0b3JhdGlvbiB0ZWNobmlxdWVzLiBMYWtlIHJlc3RvcmF0aW9uIHRlY2huaXF1ZXMgYXJlIHJldmlld2VkLCBwcm92aWRpbmcgdW5kZXJzdGFuZGluZyBvZiB0aGUgdW5kZXJseWluZyBtZWNoYW5pc21zLiBTTUZDcyBhcmUgYXBwcm9hY2hlZCBmcm9tIGFuIGVuZ2luZWVyaW5nIHBlcnNwZWN0aXZlIGFuZCBhIGZvY3VzIG9uIGxha2UgcmVtZWRpYXRpb24uIFRoZSBzcGVjaWZpYyBvYmplY3RpdmVzIHdlcmUgdG8gdGVzdCBhIGxvdy1jb3N0IGVsZWN0cm9kZSBtYXRlcmlhbCBmb3IgaXRzIGR1cmFiaWxpdHksIHByYWN0aWNhbGl0eSwgYW5kIHBlcmZvcm1hbmNlLiBQZXJmb3JtYW5jZSB3YXMgYXNzZXNzZWQgYXMgdGhlIGFiaWxpdHkgdG8gbGltaXQgcGhvc3Bob3J1cyByZWxlYXNlIGZyb20gc2VkaW1lbnRzLCBpbmhpYml0IG1ldGhhbmUgcmVsZWFzZSwgYW5kIHRoZSBhZGRlZCBiZW5lZml0cyBvZiBkZWdyYWRpbmcgcG9sbHV0YW50cywgYW5kIGdlbmVyYXRpbmcgZWxlY3RyaWNpdHkuIFByb29mIG9mIGNvbmNlcHQgd2FzIHNob3duIGluIGxhYm9yYXRvcnkgcmVhY3RvcnMgKDEgTCksIHRoZSBlZmZlY3Qgb2YgdXBzY2FsaW5nIHRvIDIwMCBMIGluIG5hdHVyYWwgZW52aXJvbm1lbnQgd2FzIHN0dWRpZWQgaW4gb3V0ZG9vciBtZXNvY29zbXMsIGFuZCB0aGVuIHRlc3RlZCBhdCBwaWxvdCBzY2FsZSBpbiBhIGV1dHJvcGhpYyBmcmVzaHdhdGVyIGxha2UuIFBpbG90IGluc3RhbGxhdGlvbnMgd2VyZSB0aGUgbGFyZ2VzdCBpbiB0aGUgd29ybGQgb2YgdGhlaXIga2luZCBhbmQgd2VyZSBvcGVyYXRlZCBmb3IgMiB5ZWFycy4gU3RhaW5sZXNzIHN0ZWVsIHdhcyBzZWxlY3RlZCBmb3IgdGVzdGluZyBhcyBlbGVjdHJvZGUgbWF0ZXJpYWwgZHVlIHRvIGl0cyBsb3cgY29zdCwgZHVyYWJpbGl0eSwgYW5kIGF2YWlsYWJpbGl0eS4gUGFpcnMgb2YgZWxlY3Ryb2RlcyB3ZXJlIGVtYmVkZGVkIGluIHRoZSBzZWRpbWVudCAodGhpcyBpcyBjYWxsZWQgdGhlIGFub2RlKSBhbmQgc3VzcGVuZGVkIGluIHRoZSBvdmVybHlpbmcgd2F0ZXIgKHRoaXMgaXMgY2FsbGVkIHRoZSBjYXRob2RlKS4gVGhlIHR3byBlbGVjdHJvZGVzIHdlcmUgY29ubmVjdGVkIGFjcm9zcyBhbiBleHRlcm5hbCByZXNpc3RvciB3aXRoIGVsZWN0cmljYWwgd2lyZXMuIE1pY3JvYmVzIGluIHRoZSBzZWRpbWVudCB1c2UgdGhlIGFub2RlIHBhaXIgYXMgZWxlY3Ryb24gYWNjZXB0b3IsIGVuYWJsaW5nIHJlc3BpcmF0aW9uIGluIHRoZSBhbm94aWMgc2VkaW1lbnQuIEVsZWN0cm9ucyB0cmF2ZWwgZnJvbSB0aGUgc2VkaW1lbnQgdmlhIHRoZSBlbGVjdHJpY2FsIGNvbm5lY3RvciB0byB0aGUgb3Zlcmx5aW5nIGNhdGhvZGUgd2hlcmUgb3h5Z2VuIGluIHRoZSB3YXRlciBpcyB0aGUgdGVybWluYWwgZWxlY3Ryb24gYWNjZXB0b3IgY29tcGxldGluZyB0aGUgY2lyY3VpdC4gQXMgYW4gYWRkZWQgYmVuZWZpdCwgZGVncmFkYXRpb24gb2YgcG9sbHV0YW50cyB1c2luZyBTTUZDIHdhcyB0ZXN0ZWQgaW4gbWFyaW5lIGhhcmJvdXIgc2VkaW1lbnRzLCBidXQgbm8gY29uY2x1c2l2ZSByZXN1bHRzIG9idGFpbmVkLiBMYWJvcmF0b3J5IHJlYWN0b3JzIHdlcmUgb3BlcmF0ZWQgZm9yIG1vcmUgdGhhbiAyMDAgZGF5cyB1c2luZyBwZXRyb2xldW0gYXMgc29sZSBjYXJib24gc291cmNlLCBjb25maXJtaW5nIG9pbCBkZWdyYWRhdGlvbiBieSBTTUZDcy4gQSByZW1vdmFsIHJhdGUgb2YgMC45IGcgcGV0cm9sZXVtIG0tMyBkLTEgcmVhY3RvciB2b2x1bWUgd2FzIGVzdGltYXRlZCBmcm9tIG1lYXN1cmVkIGVsZWN0cmljYWwgb3V0cHV0LiBUaGlzIHdhcyBkb25lIGJ5IGFzc3VtcHRpb24gb2YgdGhlIGNoZW1pY2FsIHJlYWN0aW9uIG9jY3VycmluZywgZnJvbSB0aGlzIHRoZSBudW1iZXIgb2YgZWxlY3Ryb25zIHRyYW5zZmVycmVkIGZyb20gb2lsIHRvIHdhdGVyIHdhcyBrbm93biBhbmQgY291bGQgYmUgY29ycmVsYXRlZCB0byB0aGUgbWVhc3VyZWQgZmxvdyBvZiBlbGVjdHJvbnMgKGN1cnJlbnQpIHRocm91Z2ggdGhlIGV4dGVybmFsIGNpcmN1aXQuIFN0YWlubGVzcyBzdGVlbCBlbGVjdHJvZGVzIHdlcmUgb3BlcmF0ZWQgaW4gZnJlc2h3YXRlciBsYWtlIGVudmlyb25tZW50cyBmb3IgbW9yZSB0aGFuIDQgeWVhcnMgd2l0aCBtaW5pbWFsIGRhbWFnZSB0byB0aGUgbWF0ZXJpYWwuIFRoZSBlbGVjdHJvZGUgaXMgdGhlIG1haW4gbWF0ZXJpYWwgY29zdCBpbiBzdWNoIHN5c3RlbXMuIEhvd2V2ZXIsIEFJU0kzMTYgc3RhaW5sZXNzIHN0ZWVsIHNpbWlsYXIgdG8gd2hhdCB3YXMgdXNlZCBpbiB0aGlzIHRoZXNpcyB3YXMgYXQgdGhlIHRpbWUgb2Ygd3JpdGluZyBhdmFpbGFibGUgYXQgYnVsayBjb3N0IG9mIDAuMDkxIOKCrCBtLTIsIGNvcnJlc3BvbmRpbmcgdG8gOTEwIOKCrCBhY3JlLTEsIHNpZ25pZmljYW50bHkgbG93ZXIgdGhhbiBvdGhlciBjb21tb24gbWF0ZXJpYWxzLiBDYXRob2RlcyBhdCBsYXJnZSBzY2FsZSBjb3VsZCBiZSBtb3ZlZCBob3Jpem9udGFsbHkgYW5kIHZlcnRpY2FsbHkgd2l0aCBtaW5vciBpbXBhY3QgdG8gcGVyZm9ybWFuY2UuIFBpbG90IHNjYWxlIFNNRkNzIGdlbmVyYXRlZCAwLjM4MyBtVyBtLTIgc3VzdGFpbmVkIGFuZCAyNy40IG1XIG0tMiBwZWFrIG91dHB1dC4gVGhlIHN5c3RlbSB3YXMgbGltaXRlZCBieSB0aGUgY2F0aG9kZSBzdXJmYWNlIGFyZWEsIG1lYW5pbmcgbXVsdGktbGF5ZXIgY2F0aG9kZXMgc2hvdWxkIGJlIHRlc3RlZCBpbiB0aGUgZnV0dXJlIGlmIGVsZWN0cmljYWwgcG93ZXIgcHJvZHVjdGlvbiBpcyB0aGUgZ29hbC4gVGhlIGNhdGhvZGUgc2hvdWxkIGJlIHBsYWNlZCBhdCBhIGRlcHRoIHRoYXQgZW5zdXJlcyBoaWdoIGRpc3NvbHZlZCBveHlnZW4gY29uY2VudHJhdGlvbnMgdG8gYXZvaWQgdGhlIHN5c3RlbSB0byBiZSBsaW1pdGVkLiBIb3dldmVyLCB0b28gaGlnaCBlbGVjdHJpY2l0eSBnZW5lcmF0aW9uIHdhcyBmb3VuZCBjb3VudGVycHJvZHVjdGl2ZSBmb3IgcmVtZWRpYXRpb24gcHVycG9zZXMgYmVjYXVzZSB0aGlzIHdvdWxkIGluY3JlYXNlIHNlZGltZW50IHR1cm5vdmVyIHJhdGUsIHJlbGVhc2luZyBwb2xsdXRhbnRzIHN0b3JlZCBpbiB0aGUgc2VkaW1lbnRzLiBQIGJpbmRpbmcgd2FzIG5vdCBmb3VuZCB0byByZXF1aXJlIGhpZ2ggdHVybm92ZXIgb2Ygc2VkaW1lbnQgb3JnYW5pYyBtYXR0ZXIuIFRoZSB0dXJub3ZlciBjYW4gYmUgY29udHJvbGxlZCBieSBjaGFuZ2luZyB0aGUgZXh0ZXJuYWwgcmVzaXN0YW5jZS4gSGlnaCByZXNpc3RhbmNlIGxpbWl0cyB0aGUgZWxlY3RyaWNhbCBwZXJmb3JtYW5jZSwgbG93IHJlc2lzdGFuY2UgaW5jcmVhc2VzIHNlZGltZW50IHR1cm5vdmVyIGFuZCByaXNrcyBkZXN0YWJpbGl6aW5nIHRoZSBzeXN0ZW0uIFRvIGFjaGlldmUgcmVkdWNlZCBjb25jZW50cmF0aW9ucyBvZiBwaG9zcGhhdGUgaW4gdGhlIHdhdGVyIGNvbHVtbiwgYW5vZGVzIG11c3QgYmUgcGxhY2VkIGluIHRoZSBzZWRpbWVudCBzdXJmYWNlIHNpbmNlIGEgZmV3IG1pbGxpbWV0cmVzIG9mIHNlZGltZW50IG1heSBjb250YWluIGFuZCByZWxlYXNlIGVub3VnaCBQIHRvIHN1c3RhaW4gZXV0cm9waGljYXRpb24uIFBob3NwaGF0ZSB3YXMgbWVhc3VyZWQgaW4gdGhlIHdhdGVyIGNvbHVtbiBuZWFyIHRoZSBzZWRpbWVudCwgaW4gc3lzdGVtcyB3aXRoIGFuZCB3aXRob3V0IGVsZWN0cm9kZXMuIFBob3NwaGF0ZSBjb25jZW50cmF0aW9ucyB3ZXJlIHN1Y2Nlc3NmdWxseSByZWR1Y2VkIGJ5IHVwIHRvIDk1ICUgaW4gMi1jaGFtYmVyIGxhYm9yYXRvcnkgcmVhY3RvcnMsIGFuZCBieSB1cCB0byA5OCAlIGluIHRoZSBwaWxvdCBpbnN0YWxsYXRpb24uIEluIHRoZSBwaWxvdCBleHBlcmltZW50LCBwaG9zcGhhdGUgY29uY2VudHJhdGlvbnMgYWJvdmUgZWxlY3Ryb2RlcyB3ZXJlIGxvd2VyIGluIGFsbCBtZWFzdXJlbWVudHMsIGNvbXBhcmVkIHRvIGNvbnRyb2xzLiBTZWRpbWVudCByZWRveCBwb3RlbnRpYWxzIGluY3JlYXNlZCBmcm9tIC00MDAgbVYgdnMgc3RhbmRhcmQgaHlkcm9nZW4gZWxlY3Ryb2RlIChTSEUpIHRvIC0xNzUgbVYgdnMgU0hFLCBjb3JyZXNwb25kaW5nIHRvIGEgbW92ZSBmcm9tIGEgbWV0aGFub2dlbmV0aWMgZG9tYWluIHRvIHN1bHBodXIgcmVkdWNpbmcuIEFuYWx5c2lzIG9mIHRoZSBtaWNyb2JpYWwgY29tbXVuaXR5IGluZGljYXRlZCBlbmhhbmNlZCBzdWxwaHVyIGN5Y2xpbmcgd2hlcmUgZWxlY3Ryb2RlcyBveGlkaXplIHN1bHBoaWRlIHdoaWNoIHRoZSBiYWN0ZXJpYSBhZ2FpbiByZWR1Y2UgdG8gb2J0YWluIGVuZXJneS4gSXQgd2FzIGh5cG90aGVzaXNlZCB0aGF0IFNNRkMgcHJvbW90ZSBoaWdoZXIgcmVkb3ggcG90ZW50aWFscyBieSBmYWNpbGl0YXRpbmcgb3hpZGF0aW9uIG9mIHN1bHBodXIgYW5kIGlyb24gc3BlY2llcywgaW5oaWJpdGluZyBIMlMgZm9ybWF0aW9uIGFuZCBGZSBzY2F2ZW5naW5nIGJ5IEZlUyBmb3JtYXRpb24sIHRodXMgYWxsb3dpbmcgZm9yIFAgYmluZGluZy4gQSBsb3dlciByZWxhdGl2ZSBhYnVuZGFuY2Ugb2YgbWV0aGFub2dlbnMgb24gZWxlY3Ryb2RlcyBjb21wYXJlZCB0byBjb250cm9scyBzdWdnZXN0cyBzdXBwcmVzc2lvbiBvciBvdXRjb21wZXRpbmcgb2YgbWV0aGFub2dlbnMuUXVhbnRpZmljYXRpb24gb2YgbWV0aGFuZSBwcm9kdWN0aW9uIHdhcyBkb25lIG9uIHRoZSBiYXNpcyBvZiBhIENINDpDTzIgcmF0aW8gdG8gYWxsb3cgY29tcGFyYWJpbGl0eSBiZXR3ZWVuIHN5c3RlbXMuIE1ldGhhbmUgZW1pc3Npb24gd2FzIHJlZHVjZWQgYnkgdXAgdG8gOTkgJSBhbmQgbWFpbnRhaW5lZCBpbmhpYml0aW9uIGZvciBzZXZlcmFsIG1vbnRocyBpbiBsYWJvcmF0b3J5IHN5c3RlbXMgd2l0aCBlbGVjdHJvZGVzLiBJbiB0aGUgcGlsb3QgaW5zdGFsbGF0aW9ucyBubyBnYXMgc2FtcGxlcyBjb3VsZCBiZSBjb2xsZWN0ZWQsIGFuZCBubyBDTzI6Q0g0IHJhdGlvcyB3ZXJlIG9idGFpbmVkLiBBZXJhdGlvbiBpcyB0aGUgY3VycmVudCByZXN0b3JhdGlvbiB0ZWNobmlxdWUgdGhhdCBtb3N0IGNsb3NlbHkgcmVzZW1ibGVzIHRoZSBwcm9wb3NlZCBTTUZDIHRlY2huaXF1ZS4gQWVyYXRpb24gaGFzIHByZXZpb3VzbHkgZmFpbGVkIHRvIHJlZHVjZSBQIHJlbGVhc2UgaW4gc2V2ZXJhbCBzZXBhcmF0ZSBpbnZlc3RpZ2F0aW9ucy4gU01GQ3MgYXMgYSByZW1lZGlhdGlvbiB0b29sIHdhcyBmb3VuZCB0byBvZmZlciBzZXZlcmFsIGJlbmVmaXRzIG92ZXIgY29udmVudGlvbmFsIG1ldGhvZHM6IFNNRkNzIG9mZmVycyBjb250aW51b3VzIGVmZmVjdCwgd2hpbGUgb3RoZXIgdGVjaG5pcXVlcyByZXF1aXJlIGNvbnRpbnVvdXMgb3IgcmVwZWF0ZWQgdHJlYXRtZW50LiBTTUZDcyBjYW4gYmUgY29udHJvbGxlZCBpbiByZWFsLXRpbWUgYnkgYWx0ZXJpbmcgdGhlIGV4dGVybmFsIHJlc2lzdG9yIGFuZCBjYW4gYmUgZWFzaWx5IG1vbml0b3JlZCBieSBtZWFzdXJpbmcgdGhlIHByb2R1Y2VkIGVsZWN0cmljYWwgb3V0cHV0LiBJZiBpbXBsZW1lbnRlZCBhdCBsYXJnZSBzY2FsZSBpdCBpcyBleHBlY3RlZCB0aGF0IHRoZSBlbGVjdHJvZGUgZmllbGRzIHdpbGwgaGVscCBtYWludGFpbiBsb3cgcGhvc3BoYXRlIGNvbmNlbnRyYXRpb25zIGluIHRoZSB3YXRlciBmb3IgZGVjYWRlcyB3aXRoIG1pbmltYWwgaW50ZXJ2ZW50aW9uLCBhbmQgYWZ0ZXIgc29tZSB0aW1lLCBiZW50aGljIHBsYW50cyBhcmUgZXhwZWN0ZWQgdG8gcmV0dXJuLCB0aHVzIG1haW50YWluaW5nIHRoZSBiYWxhbmNlIGhlbmNlIGZvcnRoLiIsInVybCI6Imh0dHBzOi8vbG9jYWwuZm9yc2tuaW5nc3BvcnRhbC5kay9sb2NhbC9ka2ktY2dpL3dzL2NyaXMtbGluaz9zcmM9ZHR1JmlkPWR0dS1kNjZkMzM3MC0yNTZmLTQ4ODUtYTA5ZS0zMzZhOTI0ZjRiZWQmdGk9U2VkaW1lbnQlMjBtaWNyb2JpYWwlMjBmdWVsJTIwY2VsbHMlMjBmb3IlMjBsYWtlJTIwcmVzdG9yYXRpb24iLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vYmFja2VuZC5vcmJpdC5kdHUuZGsvd3MvZmlsZXMvMjQ4MDY5NTg5L1RoZXNpc19vbmxpbmVfdmVyc2lvbl9LYXJsX0hheHRoYXVzZW4ucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjEtMDEtMDEiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA4IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJSZXNlYXJjaCBQb3J0YWwgRGVubWFyayIsImltcGFjdEZhY3RvciI6IjAuMDQifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjo3Nn19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJUZWNobmljYWwgVW5pdmVyc2l0eSBvZiBEZW5tYXJrIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjowLCJjaXRlZEJ5Q291bnQiOjF9fSwicXVvdGUiOiJFeHRlcm5hbCBpbnB1dHMgaW5jcmVhc2UgUCBjb25jZW50cmF0aW9uIGluICB0aGUgd2F0ZXIsIGNhdXNpbmcgYWxnYWwgYmxvb21zIHRoYXQgc2hhZG93IHRoZSBsYWtlIGJvdHRvbSwgYW5kIGJlbnRoaWMgbWFjLXJvcGh5dGVzIHJlY2VkZS4gUHJlZGF0b3J5IGZpc2ggYXJlIHVuYWJsZSB0byBodW50LCBhbmQgdGhlaXIgZm9vZCBzb3VyY2UgaXMgIGRpc3J1cHRlZCBzdWNoIGFzIGZpc2ggZmVlZGluZyBvbiBiZW50aGljIGludmVydGVicmF0ZXMgYW5kIGJvdHRvbS1kd2VsbGluZyBhbmltYWxzIGFyZSBzdWZmb2NhdGVkIChWYW5kZXIgWmFuZGVuIGFuZCBWYWRlYm9uY29ldXIgMjAwMiksIHNvIHRoYXQgcGxhbmt0aXZvcm91cyBmaXNoIHNwZWNpZXMgZG9taW5hdGUuIFRoaXMgaW5jcmVhc2VzIHByZWRhdGlvbiBvbiB6b28gcGxhbmt0b24gIGFuZCByZWR1Y2VzIGdyYXppbmcgb24gYWxnYWUuIEFsZ2FlIHByb2xpZmVyYXRlLCBkaWUsIGFuZCBzaW5rIHRvIHRoZSBib3R0b20sICBjb25zdW1pbmcgb3h5Z2VuIHdoZW4gZGVncmFkZWQsIHdoaWNoIGxlYWRzIHRvIG94eWdlbiBkZWZpY2l0IGluIHRoZSBib3QtdG9tLCBraWxsaW5nIGZpc2ggKFd1cnRzYmF1Z2ggZXQgYWwuIDIwMTkpLiBGaWd1cmUgMjogVmljaW91cyBjeWNsZXMgb2YgZXV0cm9waGljYXRpb24uIExlZnQ6IEludGVybGlua2VkIHByb2Nlc3NlcyBzdXN0YWluaW5nIGV1dHJvcGhpLWNhdGlvbiBpbiBhIGxha2UuIEJsdWU6IHByb2Nlc3NlcyBpbiB3YXRlciwgZ3JlZW46IHByb2Nlc3NlcyBpbnZvbHZpbmcgbWljcm9hbGdhZSwgZ3JleTogc2VkLWltZW50IHJlbGF0ZWQgcHJvY2Vzc2VzLCB5ZWxsb3c6IHByb2Nlc3NlcyBpbnZvbHZpbmcgZmF1bmEuIFJpZ2h0LCBkYXNoZWQgbGluZTogVGhlIG9yZGVyIGluIHdoaWNoIGVsZWN0cm9uIGFjY2VwdG9ycyBhcmUgY29uc3VtZWQgaW4gdGhlIGFic2VuY2Ugb2YgTzIgc2hvd24gYXMgY29uY2VudHJhdGlvbiB2ZXJzdXMgIGRlcHRoIGluIHNlZGltZW50LCBhbmQgYXQgd2hpY2ggcG90ZW50aWFsIHRoaXMgb2NjdXJzIChCZXJuZXIgMTk4MSwgYnkgY291cnRlc3kgb2YgdGhlIHNvY2lldHkgIGZvciBzZWRpbWVudGFyeSBnZW9sb2d5KS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjE3LCJiYm94Ijp7InRvcCI6NzkuOTMzMTUxMjQ1MTE3MTksImxlZnQiOjE0LjI5NTUwMzYxNjMzMzAwOCwid2lkdGgiOjcxLjkzODkzNDMyNjE3MTg4LCJoZWlnaHQiOjIuMDUzNDc3Mjg3MjkyNDgwNX19LHsicGFnZU51bWJlciI6MTcsImJib3giOnsidG9wIjo4MS45NzEzNDM5OTQxNDA2MiwibGVmdCI6MTQuMjk1NTAzNjE2MzMzMDA4LCJ3aWR0aCI6NzEuMzMzMDAwMTgzMTA1NDcsImhlaWdodCI6Mi4wNTM0NzcyODcyOTI0ODA1fX0seyJwYWdlTnVtYmVyIjoxNywiYmJveCI6eyJ0b3AiOjg0LjAyMzc5NjA4MTU0Mjk3LCJsZWZ0IjoxNC4yOTU1MDM2MTYzMzMwMDgsIndpZHRoIjo3MS44MTIwMTkzNDgxNDQ1MywiaGVpZ2h0IjoyLjA1MzQ3NzI4NzI5MjQ4MDV9fSx7InBhZ2VOdW1iZXIiOjE3LCJiYm94Ijp7InRvcCI6ODYuMDYxOTk2NDU5OTYwOTQsImxlZnQiOjE0LjI5NTUwMzYxNjMzMzAwOCwid2lkdGgiOjcxLjk1NzM1OTMxMzk2NDg0LCJoZWlnaHQiOjIuMDUzNDc3Mjg3MjkyNDgwNX19LHsicGFnZU51bWJlciI6MTgsImJib3giOnsidG9wIjo5LjkyNjQ4NzkyMjY2ODQ1NywibGVmdCI6MTQuMjk1NTAzNjE2MzMzMDA4LCJ3aWR0aCI6NzEuOTc0MDYwMDU4NTkzNzUsImhlaWdodCI6MTAuMjM0Nzc3NDUwNTYxNTIzfX0seyJwYWdlTnVtYmVyIjoxOCwiYmJveCI6eyJ0b3AiOjQ0LjczNDA0MzEyMTMzNzg5LCJsZWZ0IjoxNC4yOTU1MDM2MTYzMzMwMDgsIndpZHRoIjo3MS44OTM0MzI2MTcxODc1LCJoZWlnaHQiOjEwLjQwMzI4OTc5NDkyMTg3NX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjkzIn19LHsib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1pY2FoIE0uIiwiZmFtaWx5IjoiU3dhbm4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQWxpY2lhIiwiZmFtaWx5IjoiQ29ydMOpcyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBbGV4YW5kZXIgTC4iLCJmYW1pbHkiOiJGb3JyZXN0In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5pY2hvbGFzIFQuIiwiZmFtaWx5IjoiRnJhbXN0ZWQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU3RldmVuIiwiZmFtaWx5IjoiU2Fkcm8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiR2VvZmZyZXkiLCJmYW1pbHkiOiJTY2hsYWRvdyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBbmdlbGEgRGUiLCJmYW1pbHkiOiJQYWxtYeKAkERvdyJ9fV0sImlkcyI6eyJpc3NuIjpbIjEwMTUtMTYyMSIsIjE0MjAtOTA1NSJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMTAwNy9zMDAwMjctMDI0LTAxMDQ1LTIiLCJvcGVuYWxleCI6Ilc0MzkyNzEzMzk5IiwiZXhhIjoiaHR0cHM6Ly9saW5rLnNwcmluZ2VyLmNvbS9hcnRpY2xlLzEwLjEwMDcvczAwMDI3LTAyNC0wMTA0NS0yP2Vycm9yPWNvb2tpZXNfbm90X3N1cHBvcnRlZCZjb2RlPTgzY2Y4ZDg4LWM5YTAtNGNkZC04MjU2LTFhODVlNmFiZGFjMyJ9LCJ0aXRsZSI6IkludGVybmFsIHBob3NwaG9ydXMgbG9hZGluZyBhbHRlcnMgbnV0cmllbnQgbGltaXRhdGlvbiBhbmQgY29udHJpYnV0ZXMgdG8gY3lhbm9iYWN0ZXJpYWwgYmxvb21zIGluIGEgcG9seW1pY3RpYyBsYWtlIiwiYWJzdHJhY3QiOiJBYnN0cmFjdCBDbGVhciBMYWtlLCBhIG1lZGl1bS1zaXplZCBoeXBlcmV1dHJvcGhpYywgcG9seW1pY3RpYyBsYWtlIGluIG5vcnRoZXJuIENhbGlmb3JuaWEsIGhhcyBoYWQgcmVjdXJyaW5nIGhhcm1mdWwgY3lhbm9iYWN0ZXJpYSBibG9vbXMgKEhDQnMpIGZvciBvdmVyIGEgY2VudHVyeSBkZXNwaXRlIHJlZHVjdGlvbnMgaW4gZXh0ZXJuYWwgcGhvc3Bob3J1cyAoUCkgbG9hZGluZ3MuIEludGVybmFsIFAgbG9hZGluZyBjYW4gYWx0ZXIgbnV0cmllbnQgYXZhaWxhYmlsaXR5IGFuZCBsaW1pdGF0aW9uIHN1cHBvcnRpbmcgSENCcyBidXQgaXMgcmFyZWx5IHF1YW50aWZpZWQgb3IgY29tcGFyZWQgd2l0aCBleHRlcm5hbCBsb2Fkcy4gV2UgaGF2ZSBxdWFudGlmaWVkIGV4dGVybmFsIFAgbG9hZHMgZnJvbSAyMDE5IHRvIDIwMjEgZm9yIHRoZSB0aHJlZSBtYWluIHRyaWJ1dGFyaWVzIChhY2NvdW50aW5nIGZvciA0NiUgb2YgdGhlIGZsb3cpIGFuZCBpbnRlcm5hbCBsb2FkaW5ncyB1c2luZyB0d28gbWV0aG9kczogYSBQIG1hc3MgYmFsYW5jZSBhbmQgbW9kZWxlZCByZWxlYXNlIHJhdGVzIG9mIHNvbHVibGUgcmVhY3RpdmUgcGhvc3Bob3J1cyBmcm9tIG94aWMgYW5kIGFub3hpYyBzZWRpbWVudHMuIEluIGFkZGl0aW9uLCB3ZSBjb21iaW5lZCBoaWdoLWZyZXF1ZW5jeSBpbiBzaXR1IG1lYXN1cmVtZW50cyBvZiB3YXRlciB0ZW1wZXJhdHVyZSBhbmQgZGlzc29sdmVkIG94eWdlbiwgZGlzY3JldGUgZ3JhYiBzYW1wbGluZyBmb3IgbnV0cmllbnQgY2hlbWlzdHJ5LCBhbmQgcmVtb3RlIHNlbnNpbmcgdG8gZXhwbG9yZSB0aGUgcG90ZW50aWFsIGRyaXZlcnMgb2YgdGhlIG9ic2VydmVkIHZhcmlhYmlsaXR5IGFuZCBwcm92aWRlIGEgY29tcHJlaGVuc2l2ZSB2aWV3IG9mIHRoZSBzcGF0aW90ZW1wb3JhbCBkeW5hbWljcyBvZiBIQ0JzLiBCeSB1bmRlcnN0YW5kaW5nIHRoZSByZWxhdGl2ZSBjb250cmlidXRpb24gb2YgZXh0ZXJuYWwgYW5kIGludGVybmFsIG51dHJpZW50IGxvYWRpbmdzIGFuZCB0aGUgcmVsYXRpb25zaGlwIGJldHdlZW4gZW52aXJvbm1lbnRhbCBwYXJhbWV0ZXJzIGFuZCBIQ0JzLCBpbnRlcmFubnVhbCBibG9vbSB2YXJpYWJpbGl0eSBjYW4gYmUgYmV0dGVyIHByZWRpY3RlZC4gQ29tcGFyYXRpdmUgZXN0aW1hdGVzIG9mIGV4dGVybmFsIGFuZCBpbnRlcm5hbCBwaG9zcGhvcnVzIGxvYWRpbmcgaW5kaWNhdGUgdGhhdCBpbnRlcm5hbCBzb3VyY2VzIGFjY291bnRlZCBmb3IgNzDigJM5NSUgb2YgdGhlIHRvdGFsIFAgaW5wdXQgaW50byB0aGUgc3lzdGVtIGR1cmluZyB0aGUgc3R1ZHkgcGVyaW9kLiBDb250cmFyeSB0byBvdGhlciBsYWtlcywgdGhlIGludGVuc2l0eSBvZiB0aGUgc3VtbWVyIGJsb29tIHNlYXNvbiB3YXMgY29ycmVsYXRlZCB0byB0aGUgdGltaW5nIGFuZCBkdXJhdGlvbiBvZiBhbm94aWEgcmF0aGVyIHRoYW4gdGhlIG1hZ25pdHVkZSBvZiBzcHJpbmcgcnVub2ZmLiBJbnRlcm5hbGx5IHJlbGVhc2VkIFAgc2hpZnRlZCB0aGUgc3lzdGVtIGZyb20gcGhvc3Bob3J1cyB0byBuaXRyb2dlbiBsaW1pdGF0aW9uIGR1cmluZyB0aGUgc3VtbWVyLCBwb3RlbnRpYWxseSBmYXZvcmluZyB0aGUgcHJvbGlmZXJhdGlvbiBvZiBuaXRyb2dlbi1maXhpbmcgY3lhbm9iYWN0ZXJpYS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAwNy9zMDAwMjctMDI0LTAxMDQ1LTIiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vbGluay5zcHJpbmdlci5jb20vY29udGVudC9wZGYvMTAuMTAwNy9zMDAwMjctMDI0LTAxMDQ1LTIucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjQtMDMtMTIiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTExIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJBcXVhdGljIFNjaWVuY2VzIiwiaXNzdWUiOjIsInZvbHVtZSI6ODYsImltcGFjdEZhY3RvciI6IjIuODQifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiU3ByaW5nZXIgU2NpZW5jZStCdXNpbmVzcyBNZWRpYSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NzUsImNpdGVkQnlDb3VudCI6MjF9fSwicXVvdGUiOiJDb250cmFyeSB0byBvdGhlciBsYWtlcywgdGhlIGludGVuc2l0eSBvZiB0aGUgc3VtbWVyIGJsb29tIHNlYXNvbiB3YXMgY29ycmVsYXRlZCB0byB0aGUgdGltaW5nIGFuZCBkdXJhdGlvbiBvZiBhbm94aWEgcmF0aGVyIHRoYW4gdGhlIG1hZ25pdHVkZSBvZiBzcHJpbmcgcnVub2ZmLiBJbnRlcm5hbGx5IHJlbGVhc2VkIFAgc2hpZnRlZCB0aGUgc3lzdGVtIGZyb20gcGhvc3Bob3J1cyB0byBuaXRyb2dlbiBsaW1pdGF0aW9uIGR1cmluZyB0aGUgc3VtbWVyLCBwb3RlbnRpYWxseSBmYXZvcmluZyB0aGUgcHJvbGlmZXJhdGlvbiBvZiBuaXRyb2dlbi1maXhpbmcgY3lhbm9iYWN0ZXJpYS4gLSBbQ2FsY2l1bSBhbmQgcGhvc3BoYXRlIG1ldGFib2xpYyBkaXNvcmRlcnNdKGh0dHBzOi8vbGluay5zcHJpbmdlci5jb20vc3ViamVjdHMvY2FsY2l1bS1hbmQtcGhvc3BoYXRlLW1ldGFib2xpYy1kaXNvcmRlcnMpIEV1dHJvcGhpY2F0aW9uIHBvc2VzIGEgZ3JhdmUgdGhyZWF0IHRvIGxlbnRpYyBlY29zeXN0ZW1zIGdsb2JhbGx5LCBwcm9tb3RpbmcgdGhlIHByb2xpZmVyYXRpb24gb2YgaGFybWZ1bCBjeWFub2JhY3RlcmlhIGJsb29tcyAoSENCcykgdGhhdCByZWR1Y2UgYXF1YXRpYyBiaW9kaXZlcnNpdHksIHlpZWxkaW5nIGluY3JlYXNlZCB3YXRlciB0cmVhdG1lbnQgY29zdHMsIGFuZCB0aHJlYXRlbmluZyBodW1hbiBoZWFsdGggdGhyb3VnaCBibG9vbS1wcm9kdWNlZCBjeWFub3RveGlucyAoQXLDoW96IGV0IGFsLiJ9XQ%3D%3D.

In Indian contexts, this infrastructure gap manifests acutely in lakes encircled by burgeoning urban sprawl. Bellandur Lake in Bengaluru, receiving 500 MLD of partly treated sewage, exemplifies perennial froth from Microcystis scums, with DO crashes triggering fish kills and >80% biodiversity loss (Jilbert et al., 2020; Nair & Nayak, 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRvbSIsImZhbWlseSI6IkppbGJlcnQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUmFvdWzigJBNYXJpZSIsImZhbWlseSI6IkNvdXR1cmUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQnJpYW4iLCJmYW1pbHkiOiJIdXNlciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLYWxldmkiLCJmYW1pbHkiOiJTYWxvbmVuIn19XSwiaWRzIjp7Imlzc24iOlsiMDAxOC04MTU4IiwiMTU3My01MTE3Il0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDA3L3MxMDc1MC0wMjAtMDQ0NTcteCIsIm1hZyI6IjMxMDA0NTkwNTAiLCJvcGVuYWxleCI6IlczMTAwNDU5MDUwIn0sInRpdGxlIjoiUHJlZmFjZTogUmVzdG9yYXRpb24gb2YgZXV0cm9waGljIGxha2VzOiBjdXJyZW50IHByYWN0aWNlcyBhbmQgZnV0dXJlIGNoYWxsZW5nZXMiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAwNy9zMTA3NTAtMDIwLTA0NDU3LXgiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vbGluay5zcHJpbmdlci5jb20vY29udGVudC9wZGYvMTAuMTAwNy9zMTA3NTAtMDIwLTA0NDU3LXgucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjAtMTEtMTYiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA3IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJIeWRyb2Jpb2xvZ2lhIiwiaXNzdWUiOjIxLCJ2b2x1bWUiOjg0NywiaW1wYWN0RmFjdG9yIjoiMy4wMCJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjQzNDMsImxhc3RQYWdlIjo0MzU3fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlNwcmluZ2VyIFNjaWVuY2UrQnVzaW5lc3MgTWVkaWEifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjk1LCJjaXRlZEJ5Q291bnQiOjU4fX0sInF1b3RlIjoiU2ltaWxhciBkZWdyYWRhdGlvbiBvZiBmcmVzaHdhdGVyIGVjb3N5c3RlbXMgaXMgb2JzZXJ2ZWQgd29ybGR3aWRlLCB3aXRoIHRoZSB0aW1lbGluZSBvZiBodW1hbiBwcmVzc3VyZSBkaWN0YXRlZCBieSB0cmVuZHMgaW4gdXJiYW5pemF0aW9uIGFuZCBpbnRlbnNpdmUgYWdyaWN1bHR1cmUgaW4gZGlmZmVyZW50IGdlb2dyYXBoaWMgcmVnaW9ucyAoZS5nLiwgVmVubmVtbyBldCBhbC4sIDIwMDkpLiBUaGUgcHJpbmNpcGFsIGRyaXZlciBvZiBlY29zeXN0ZW0gc3RyZXNzIGluIG1vc3QgYXF1YXRpYyBzeXN0ZW1zIGlzIGV1dHJvcGhpY2F0aW9uLCB0aGUgZXhjZXNzIGdyb3d0aCBvZiBhbGdhZSBkcml2ZW4gYnkgYW50aHJvcG9nZW5pYyBpbnB1dHMgb2YgdGhlIG1hY3JvbnV0cmllbnRzIG5pdHJvZ2VuIChOKSBhbmQgcGhvc3Bob3J1cyAoUCkgKFNtaXRoLCAyMDAzOyBCaXJrIGV0IGFsLiwgMjAyMCkuIE51dHJpZW50cyBhcmUgZW1pdHRlZCBmcm9tIGJvdGggcG9pbnQgYW5kIGRpZmZ1c2Ugc291cmNlcyBpbiB0aGUgaHVtYW4gZW52aXJvbm1lbnQgYW5kIGVudGVyIGFxdWF0aWMgc3lzdGVtcyB2aWEgc3VyZmFjZSBydW5vZmYsIGFzIHdlbGwgYXMgZ3JvdW5kLXdhdGVyIGFuZCBhdG1vc3BoZXJpYyBkZXBvc2l0aW9uIChQYWVybCwgMTk5NykuIEV1dHJvcGhpY2F0aW9uIG9mdGVuIGxlYWRzIHRvIHRoZSBkb21pbmFuY2Ugb2YgcHJpbWFyeSBwcm9kdWN0aW9uIGJ5IGN5YW5vYmFjdGVyaWEsIGluY2x1ZGluZyB0b3hpYyBnZW5lcmEgc3VjaCBhcyBEb2xpY2hvc3Blcm11bSwgQXBoYW5pem9tZW5vbiwgYW5kIE1pY3JvY3lzdGlzIChIdWlzbWFuIGV0IGFsLiwgMjAxOCksIGFzIHdlbGwgYXMgb3h5Z2VuIGRlcGxldGlvbiBhbmQgaGFiaXRhdCBkZWdyYWRhdGlvbiBpbiBsYWtlcyAoQXJlbmQgZXQgYWwuLCAyMDExKS4gQmVjYXVzZSBsYWtlcyBwcm92aWRlIGEgcmFuZ2Ugb2YgY3J1Y2lhbCBlY29zeXN0ZW0gc2VydmljZXMgKFdpbHNvbiAmIENhcnBlbnRlciwgMTk5OSksIHRoZXJlIGlzIGEgZ3Jvd2luZyBuZWVkIGZvciBtYW5hZ2VtZW50IGFwcHJvYWNoZXMgdGhhdCByZXN0b3JlIHdhdGVyIHF1YWxpdHkgaW4gZnJlc2h3YXRlciBzeXN0ZW1zLiBXaXRoaW4gdGhlIGZyYW1ld29yayBvZiBVbml0ZWQgTmF0aW9ucyBTdXN0YWluYWJsZSBEZXZlbG9wLW1lbnQgR29hbCA2IChDbGVhbiBXYXRlciBhbmQgU2FuaXRhdGlvbiksIHNldmVyYWwgZGlzdGluY3QgdGFyZ2V0cyByZWxhdGUgdG8gdGhlIGltcHJvdmVtZW50IG9mIHdhdGVyIHF1YWxpdHkgYW5kIHByb3RlY3Rpb24gb2YgYXF1YXRpYyBlY29zeXN0ZW1zIGJ5IDIwMzAgKFVuaXRlZCBOYXRpb25zLCAyMDE1KS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyOC4yOTg4NjQzNjQ2MjQwMjMsImxlZnQiOjguNTQ5MjA1NzgwMDI5Mjk3LCJ3aWR0aCI6MzkuMzcxNDk0MjkzMjEyODksImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MjkuOTkxMTM0NjQzNTU0Njg4LCJsZWZ0Ijo4LjU0OTIwNTc4MDAyOTI5Nywid2lkdGgiOjM5LjM2NTExOTkzNDA4MjAzLCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMxLjY4MzQwNjgyOTgzMzk4NCwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNjk2NjMyMzg1MjUzOSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozMy4zNjc5NzMzMjc2MzY3MiwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNzU2NzkwMTYxMTMyOCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNS4wNjAyNDkzMjg2MTMyOCwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNzA5NjQwNTAyOTI5NywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNi43NTI1MTc3MDAxOTUzMSwibGVmdCI6OC41NDkyMDg2NDEwNTIyNDYsIndpZHRoIjozOS4zNzY5NjA3NTQzOTQ1MywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC40MzcwODgwMTI2OTUzMSwibGVmdCI6OC41NDkyMDg2NDEwNTIyNDYsIndpZHRoIjozOS4zNjY1Njk1MTkwNDI5NywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0MC4xMjkzNTYzODQyNzczNDQsImxlZnQiOjguNTQ5MjA4NjQxMDUyMjQ2LCJ3aWR0aCI6MzkuMzY0Mzc5ODgyODEyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0MS44MjE2MjQ3NTU4NTkzNzUsImxlZnQiOjguNTQ5MjA4NjQxMDUyMjQ2LCJ3aWR0aCI6MzkuMzY1NjczMDY1MTg1NTUsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDMuNTA2MTk1MDY4MzU5Mzc1LCJsZWZ0Ijo4LjU0OTM4OTgzOTE3MjM2Mywid2lkdGgiOjM5LjM3MDM5MTg0NTcwMzEyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0NS4xOTg0NjM0Mzk5NDE0MDYsImxlZnQiOjguNTQ5NTcyOTQ0NjQxMTEzLCJ3aWR0aCI6MzkuMzY1ODQ0NzI2NTYyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ni44OTA3MzU2MjYyMjA3LCJsZWZ0Ijo4LjU0OTU3Mjk0NDY0MTExMywid2lkdGgiOjM5LjM3Mjk1NTMyMjI2NTYyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0OC41NzUzMDIxMjQwMjM0NCwibGVmdCI6OC41NDk1NzI5NDQ2NDExMTMsIndpZHRoIjozOS4zNzI0MjEyNjQ2NDg0NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1MC4yNjc1NzgxMjUsImxlZnQiOjguNTQ5NTc2NzU5MzM4Mzc5LCJ3aWR0aCI6MzkuMzcxMTI0MjY3NTc4MTI1LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjUxLjk1OTg0MjY4MTg4NDc2NiwibGVmdCI6OC41NDk1NzY3NTkzMzgzNzksIndpZHRoIjozOS4zNjc4MzU5OTg1MzUxNTYsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTMuNTc0MTE5NTY3ODcxMDk0LCJsZWZ0Ijo4LjU0OTU3Njc1OTMzODM3OSwid2lkdGgiOjM5LjM3MDc3NzEzMDEyNjk1LCJoZWlnaHQiOjEuMzM2ODk1NTg1MDYwMTE5Nn19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjU1LjI2NjM5MTc1NDE1MDM5LCJsZWZ0Ijo4LjU0OTc2MDgxODQ4MTQ0NSwid2lkdGgiOjM5LjM2NjU4MDk2MzEzNDc2NiwiaGVpZ2h0IjoxLjMzNjg5NTU4NTA2MDExOTZ9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1Ny4wMjg5NTM1NTIyNDYwOTQsImxlZnQiOjguNTQ5OTQ0ODc3NjI0NTEyLCJ3aWR0aCI6MzkuMzYyMDI2MjE0NTk5NjEsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTguNzEzNTIzODY0NzQ2MDk0LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjkuMjk5OTgyMDcwOTIyODUyLCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjYwLjQwNTc5MjIzNjMyODEyNSwibGVmdCI6MTAuNjIyNDU1NTk2OTIzODI4LCJ3aWR0aCI6MzcuMjg3ODY4NDk5NzU1ODYsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NjIuMDk4MDYwNjA3OTEwMTU2LCJsZWZ0Ijo4LjU0OTk0MzkyMzk1MDE5NSwid2lkdGgiOjM5LjM3MzMyNTM0NzkwMDM5LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjYzLjc4MjYzMDkyMDQxMDE1NiwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzMxMjY5ODM2NDI1OCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2NS40NzQ5MDY5MjEzODY3MiwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNjUxMTk5MzQwODIwMywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2Ny4xNjcxNzUyOTI5Njg3NSwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNjk0ODAxMzMwNTY2NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2OC43ODE0NDgzNjQyNTc4MSwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzczMzg0MDk0MjM4MywiaGVpZ2h0IjoxLjMzNjg5MTUzMTk0NDI3NX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjcwLjU0NDAxMzk3NzA1MDc4LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjM5LjM2OTMwODQ3MTY3OTY5LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjcyLjIzNjI4OTk3ODAyNzM0LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjM5LjM3MjAzMjE2NTUyNzM0NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo3My45MjA4NDUwMzE3MzgyOCwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzEzMDczNzMwNDY4NzUsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNDM0NCJ9fSx7Im9yaWdpbiI6NCwid29yayI6eyJ0eXBlIjo4LCJzdWJ0eXBlIjp7Im9uZW9mS2luZCI6ImFydGljbGVUeXBlIiwiYXJ0aWNsZVR5cGUiOjF9LCJjb250cmlidXRvcnMiOlt7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJOYXZ5YSBWaWtyYW1hbiIsImZhbWlseSI6Ik5haXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUHJhdGVlcCBLdW1hciIsImZhbWlseSI6Ik5heWFrIn19XSwiaWRzIjp7Imlzc24iOlsiMjI5Ni03NzQ1Il0sImlzYm4iOltdLCJkb2kiOiIxMC4zMzg5L2ZtYXJzLjIwMjMuMTA4NzI5NiIsIm9wZW5hbGV4IjoiVzQzODI3MjMxMzMifSwidGl0bGUiOiJVbmNvdmVyaW5nIHdhdGVyIHF1YWxpdHkgYW5kIGV2YWx1YXRpbmcgdnVsbmVyYWJpbGl0aWVzIG9mIHNtYWxsLXNjYWxlIGZpc2hlcmllcyBpbiBDaGlsaWthIExhZ29vbiwgSW5kaWEiLCJhYnN0cmFjdCI6IlNtYWxsLXNjYWxlIGZpc2hlcmllcyAoU1NGcykgcGxheSBhIHN0cm9uZyByb2xlIGluIHN1c3RhaW5pbmcgbWlsbGlvbnMgb2YgbGl2ZWxpaG9vZHMsIGZvb2Qgc2VjdXJpdHksIG51dHJpdGlvbiwgYW5kIGluY29tZSBnbG9iYWxseSBidXQgdGhlIGZpc2hlcnMgZW5nYWdlZCBpbiB0aGlzIHNlY3RvciBzaW11bHRhbmVvdXNseSBleHBlcmllbmNlIGhpZ2ggbGV2ZWxzIG9mIHZ1bG5lcmFiaWxpdHkgYW5kIHByb2Nlc3NlcyBvZiBtYXJnaW5hbGlzYXRpb24uIFNldmVyYWwgZmFjdG9ycyBhcmUgYXR0cmlidXRlZCB0byB0aGUgbXVsdGlkaW1lbnNpb25hbCB2dWxuZXJhYmlsaXRpZXMgdGhlIHNtYWxsLXNjYWxlIGZpc2hlcnMgZXhwZXJpZW5jZSwgc3Bhbm5pbmcgYm90aCBuYXR1cmFsIChlLmcuLCBuYXR1cmFsIGRpc2FzdGVycywgZWNvc3lzdGVtIGNoYW5nZSkgYW5kIGFudGhyb3BvZ2VuaWMgKGUuZy4sIHBvbGljeSBjaGFuZ2UsIGh5ZHJvbG9naWNhbCBpbnRlcnZlbnRpb25zLCBhcXVhY3VsdHVyZSkgcHJlc3N1cmVzLiBXaGlsZSB0aGVyZSBpcyBtdWNoIGxpdGVyYXR1cmUgb24gdmFyaW91cyBuYXR1cmFsIGFuZCBodW1hbiBkcml2ZXJzIG9mIHZ1bG5lcmFiaWxpdHkgaW4gc21hbGwtc2NhbGUgZmlzaGVyeSBjb21tdW5pdGllcywgYW4gYWJzZW5jZSBvZiByZXNlYXJjaCBjb25uZWN0aW5nIHZ1bG5lcmFiaWxpdHkgd2l0aCB3YXRlciBxdWFsaXR5IGlzIGV2aWRlbnQuIEZpc2hlciBjb21tdW5pdGllcyBvZnRlbiB0YWxrIGFib3V0IGZpc2ggaW4gcmVsYXRpb24gdG8gdGhlIGhlYWx0aCBvZiB0aGVpciBhcXVhdGljIGhhYml0YXRzIHdoZXJlaW4gd2F0ZXIgcXVhbGl0eSBpcyBzZWVuIGFzIGEga2V5IHBhcmFtZXRlci4gVGhlIGxpbmsgYmV0d2VlbiBoZWFsdGh5IGZpc2ggYW5kIGdvb2QgcXVhbGl0eSB3YXRlciBoYXMgc2lnbmlmaWNhbnQgaW1wbGljYXRpb25zIGZvciBzdHJvbmcgYW5kIHZpYWJsZSBmaXNoaW5nIGNvbW11bml0aWVzLiBUaGlzIHBhcGVyIGV4YW1pbmVzIHRoZXNlIGxpbmtzIGZ1cnRoZXIgYnkgZm9jdXNpbmcgb24gdGhlIG5hdHVyZSBvZiB2dWxuZXJhYmlsaXRpZXMgY2F1c2VkIGJ5IHdhdGVyIHF1YWxpdHkgY2hhbmdlcyBpbiB0aGUgc21hbGwtc2NhbGUgZmlzaGVyeSBzeXN0ZW0gb2YgQ2hpbGlrYSBMYWdvb24gaW4gSW5kaWEuIFdlIHVuZGVydGFrZSBkZXRhaWxlZCBhbmFseXNpcyBvZiB0aGUgaW52YXNpdmUgc2hyaW1wIGFxdWFjdWx0dXJlIGFjdGl2aXRpZXMgYW5kIGh5ZHJvbG9naWNhbCBpbnRlcnZlbnRpb25zIGZvciBvcGVuaW5nIG9mIGEgbGFnb29uIGlubGV0IHdpdGggdGhlIEJheSBvZiBCZW5nYWwgYXMgdHdvIGRvbWluYW50IGRyaXZlcnMgYWR2ZXJzZWx5IGltcGFjdGluZyB3YXRlciBxdWFsaXR5IGFuZCBpbmNyZWFzaW5nIHZ1bG5lcmFiaWxpdGllcyBvZiB0aGUgZW50aXJlIHNtYWxsLXNjYWxlIGZpc2hlcmllcyBzb2NpYWwtZWNvbG9naWNhbCBzeXN0ZW0uIE91ciBhbmFseXNpcyBzdWdnZXN0cyB0aGF0IHRoZXJlIGFyZSBzdHJvbmcgaW50ZXJjb25uZWN0aW9ucyBiZXR3ZWVuIGNoYW5nZXMgaW4gd2F0ZXIgcXVhbGl0eSBhbmQgdGhlIGxldmVscyBvZiB2dWxuZXJhYmlsaXRpZXMgaW4gdGhlIFNTRnMgb2YgQ2hpbGlrYSBMYWdvb24uIFBvbGx1dGFudHMgc3VjaCBhcyBwZXN0aWNpZGVzLCBhbmQgb3JnYW5pYyBjb21wb3VuZHMgYWNjdW11bGF0ZSBpbiBmaXNoIHRpc3N1ZXMgYW5kIGFmZmVjdCB0aGVpciBncm93dGgsIHJlcHJvZHVjdGlvbiwgYW5kIG92ZXJhbGwgaGVhbHRoLiBUaGlzIGxlZCB0byBkZWNsaW5lcyBpbiBmaXNoIHBvcHVsYXRpb25zLCBtYWtpbmcgaXQgbW9yZSBkaWZmaWN1bHQgZm9yIGZpc2hlcnMgdG8gbWFrZSBhIGxpdmluZy4gSW4gYWRkaXRpb24gdG8gZGlyZWN0IGltcGFjdHMgb24gZmlzaCBwb3B1bGF0aW9ucywgcG9vciB3YXRlciBxdWFsaXR5IGFsc28gaGFzIGluZGlyZWN0IGVmZmVjdHMgb24gdGhlIHNvY2lhbCBhbmQgZWNvbm9taWMgdnVsbmVyYWJpbGl0eSBvZiBTU0ZzLiBGb3IgZXhhbXBsZSwgY29udGFtaW5hdGlvbiBvZiB3YXRlciBzb3VyY2VzIGxlZCB0byB0aGUgcmVkdWNlZCBudW1iZXIgb2YgZmlzaCBzcGVjaWVzIHJlZHVjaW5nIHRoZSBhbW91bnQgb2YgdGltZSBmaXNoZXJzIGNhbiBzcGVuZCBvbiBmaXNoaW5nIGFjdGl2aXRpZXMuIFRoaXMgYWxzbyBhZmZlY3RlZCB0aGUgbWFya2V0YWJpbGl0eSBvZiBmaXNoIHByb2R1Y3RzLCByZWR1Y2VkIGluY29tZSBhbmQgaW5jcmVhc2VkIHBvdmVydHkuIFRvIGZ1bGx5IHVuZGVyc3RhbmQgdGhlIGludGVyY29ubmVjdGlvbnMgYmV0d2VlbiB3YXRlciBxdWFsaXR5IGFuZCB2dWxuZXJhYmlsaXR5IGluIFNTRnMgaW4gQ2hpbGlrYSBMYWdvb24sIGl0IGlzIGltcG9ydGFudCB0byBjb25zaWRlciBib3RoIGVudmlyb25tZW50YWwgYW5kIHNvY2lhbCBmYWN0b3JzLCBhcyB3ZWxsIGFzIHRoZSBjb21wbGV4IGZlZWRiYWNrIGxvb3BzIGJldHdlZW4gdGhlc2UgZmFjdG9ycy4gVGhlIHN0dWR5IGhlbHBzIGluIGJyaWRnaW5nIGEgY3J1Y2lhbCBnYXAgaW4gb3VyIHVuZGVyc3RhbmRpbmcgb2YgdGhlIHJvbGUgb2Ygd2F0ZXIgcXVhbGl0eSBpbiB2dWxuZXJhYmlsaXR5IGFuYWx5c2lzIHdpdGhpbiByZXNvdXJjZSBkZXBlbmRlbnQgY29tbXVuaXRpZXMuIFdlIGNvbmNsdWRlIHdpdGgga2V5IGluc2lnaHRzIG9uIHBvc3NpYmxlIGNvcGluZyByZXNwb25zZXMgYW5kIGFkYXB0aXZlIGNhcGFjaXR5IG5lY2Vzc2FyeSBmb3IgdGhlIHNtYWxsLXNjYWxlIGZpc2hlcmllcyBjb21tdW5pdGllcyB0byB0cmFuc2l0aW9uIHRvd2FyZCB2aWFiaWxpdHkuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjMzODkvZm1hcnMuMjAyMy4xMDg3Mjk2IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5mcm9udGllcnNpbi5vcmcvYXJ0aWNsZXMvMTAuMzM4OS9mbWFycy4yMDIzLjEwODcyOTYvcGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjMtMDYtMjkiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTEwIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJGcm9udGllcnMgaW4gTWFyaW5lIFNjaWVuY2UiLCJ2b2x1bWUiOjEwLCJpbXBhY3RGYWN0b3IiOiI0LjI4In0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkZyb250aWVycyBNZWRpYSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NTEsImNpdGVkQnlDb3VudCI6NX19LCJxdW90ZSI6IldhdGVyIHF1YWxpdHkgb2YgQ2hpbGlrYSBMYWdvb24gaGFzIGRlZ3JhZGVkIGR1ZSB0byBhbnRocm9wb2dlbmljIGludGVydmVudGlvbnMgYW5kIG5hdHVyYWwgZm9yY2VzIG9mIGNoYW5nZS4gVmFyaW91cyBhY3Rpdml0aWVzIGluIHRoZSBDaGlsaWthIExhZ29vbiwgc3VjaCBhcyBpbmR1c3RyaWFsIHdhc3Rld2F0ZXIgZGlzcG9zYWwsIHNld2FnZSBkdW1waW5nLCBhcXVhY3VsdHVyZSwgaHlkcm9sb2dpY2FsIGludGVydmVudGlvbnMsIGFuZCBjeWNsb25lcywgYXJlIHByb2R1Y2luZyBzYWxpbml0eSBjaGFuZ2VzLCBzZWRpbWVudCBkZXBvc2l0aW9uLCBudXRyaWVudCBlbnJpY2htZW50LCBldXRyb3BoaWNhdGlvbiwgYW5kIGRlYWQgem9uZXMsIGFzIHN0YXRlZC4gRmlzaCBzdXJ2aXZhbCBpcyBhdCByaXNrIGFzIGEgcmVzdWx0IG9mIHRoZSBzaWduaWZpY2FudCByZWR1Y3Rpb24gaW4gZW52aXJvbm1lbnRhbCBjaXJjdW1zdGFuY2VzLCB3aGljaCBkaW1pbmlzaGVzIGZpc2hlcm1lbuKAmXMgaW5jb21lIGFuZCBwdXNoZXMgdGhlbSBvdXQgb2Ygc29jaWV0eS4gVGhlc2UgY2F1c2VzIGFmZmVjdGluZyB0aGUgd2F0ZXIgcXVhbGl0eSBvZiB0aGUgTGFnb29uIGRlcGljdCBtYW55IGZhY2VzIG9mIHZ1bG5lcmFiaWxpdHkgaW4gU1NGIGNvbW11bml0aWVzLiBBcyB0aGUgQ2hpbGlrYSBMYWdvb24gZW52aXJvbm1lbnQgaXMgZXhwZWN0ZWQgdG8gdW5kZXJnbyBpbmNyZWFzaW5nbHkgZHJhc3RpYyBjaGFuZ2VzIChzdWNoIGFzIGNhdGNoIGNvbXBvc2l0aW9uLCBjYXRjaCBjYXBhY2l0eSwgYmlvZGl2ZXJzaXR5IHZhcmlhdGlvbnMsIHdhdGVyIHF1YWxpdHksIGFuZCBmaXNoZXJ5IHJldmVudWVzKSwgaW50ZXJuYXRpb25hbCBwb2xpY3kgaGFzIGJlY29tZSBtb3JlIGludGVyZXN0ZWQgaW4gYWRhcHRhdGlvbiBtZWFzdXJlcy4gR2l2ZW4gaG93IGRpdmVyc2UgU1NGIGVjb3N5c3RlbXMgYXJlIGV4cGVjdGVkIHRvIGJlIGJldHdlZW4gY29udGluZW50cywgdGhlcmUgaXMgbm8g4oCcb25lIHNpemUgZml0cyBhbGzigJ0gYXBwcm9hY2guIFRoZSBwYXBlciBvdXRsaW5lZCBzZXZlcmFsIGNvcGluZyBhbmQgYWRhcHRhdGlvbiBtZWNoYW5pc21zIHRvIGltcHJvdmUgd2F0ZXIgcXVhbGl0eSBhbmQgY29udGludWUgZmlzaGluZyBpbiBhIHN1c3RhaW5hYmxlIHdheS4gQWRhcHRhdGlvbiBzdHJhdGVnaWVzIGNhbiBvY2Nhc2lvbmFsbHkgaGF2ZSB1bmZvcmVzZWVuIGNvbnNlcXVlbmNlcyBmb3IgZmlzaGVyaWVzIGFuZCBmaXNoaW5nIGNvbW11bml0aWVzLiBBbnkgdW5leHBlY3RlZCBjb25zZXF1ZW5jZXMgb2YgYWRhcHRhdGlvbiBjYW4gYmUgbWl0aWdhdGVkIHRocm91Z2ggbG9uZy10ZXJtIHBsYW5uaW5nIGFuZCBpZGVudGlmaWNhdGlvbiBvZiBmdXR1cmUgb3B0aW9ucyB0aHJvdWdoIHNjZW5hcmlvIGFuYWx5c2lzLiBUaGUgZmluZGluZ3Mgb2YgdGhpcyBwYXBlciBjYW4gYmUgdXRpbGl6ZWQgdG8gZGV2ZWxvcCBwb2xpY2llcyB0byBwcm90ZWN0IHdhdGVyIHF1YWxpdHkgYW5kIFNTRnMgaW4gYSB2YXJpZXR5IG9mIGNvYXN0YWwgbGFnb29uIHNldHRpbmdzLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyMi4yODUwMzYwODcwMzYxMzMsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40OTMwNTM0MzYyNzkzLCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyMy43NzM1MTk1MTU5OTEyMSwibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ4MzM2NDEwNTIyNDYxLCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyNS4yNTU1ODQ3MTY3OTY4NzUsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40ODk1NTkxNzM1ODM5ODQsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjI2Ljc0NDA2ODE0NTc1MTk1MywibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjUwNjgwNTQxOTkyMTg3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjguMjI2MTMzMzQ2NTU3NjE3LCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDkwNDYzMjU2ODM1OTQsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjI5LjcxNDYxNjc3NTUxMjY5NSwibGVmdCI6OC4zMzE1OTI1NTk4MTQ0NTMsIndpZHRoIjo0MC40OTQwOTg2NjMzMzAwOCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MzEuMTk2NjgzODgzNjY2OTkyLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDk0MzkyMzk1MDE5NTMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjMyLjY4NTE2NTQwNTI3MzQ0LCJsZWZ0Ijo4LjMzMTU5NjM3NDUxMTcxOSwid2lkdGgiOjQwLjQ4NzQ0NTgzMTI5ODgzLCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozNC4xNjcyMzI1MTM0Mjc3MzQsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDk3NzQ1NTEzOTE2MDE2LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozNS42NTU3MTU5NDIzODI4MSwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40OTg2MzgxNTMwNzYxNywiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MzcuMTM3NzgzMDUwNTM3MTEsImxlZnQiOjEwLjY4NTE4NjM4NjEwODM5OCwid2lkdGgiOjM4LjE0MTEwNTY1MTg1NTQ3LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozOC42MjYyNjY0Nzk0OTIxOSwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40ODcxNTIwOTk2MDkzNzUsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQwLjEwODMzMzU4NzY0NjQ4NCwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40ODk4NTY3MTk5NzA3LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo0MS41OTY4MTMyMDE5MDQzLCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4ODY2NjUzNDQyMzgzLCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo0My4wNzg4ODAzMTAwNTg1OTQsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDgxODYxMTE0NTAxOTUsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQ0LjU2NzM2MzczOTAxMzY3LCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4MzA1ODkyOTQ0MzM2LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo0Ni4wNDk0MzA4NDcxNjc5NywibGVmdCI6OC4zMzMxMDg5MDE5Nzc1MzksIndpZHRoIjo0MC41MDA3NDc2ODA2NjQwNiwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDcuNTM3OTE0Mjc2MTIzMDUsImxlZnQiOjguMzMzMTA4OTAxOTc3NTM5LCJ3aWR0aCI6NDAuNDk1NjA1NDY4NzUsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQ5LjAxOTk4MTM4NDI3NzM0NCwibGVmdCI6OC4zMzMxMDYwNDA5NTQ1OSwid2lkdGgiOjQwLjUwNjgxNjg2NDAxMzY3LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo1MC41MDg0NjQ4MTMyMzI0MiwibGVmdCI6OC4zMzMxMDYwNDA5NTQ1OSwid2lkdGgiOjQwLjQ4ODUwNjMxNzEzODY3LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo1MS45OTA1MjgxMDY2ODk0NSwibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ5NzcyNjQ0MDQyOTY5LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo1My40NzkwMTE1MzU2NDQ1MywibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjMzLjE1OTQ0NjcxNjMwODU5NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NTQuOTYxMDc4NjQzNzk4ODMsImxlZnQiOjEwLjY4MzY3MDA0Mzk0NTMxMiwid2lkdGgiOjM4LjEzNjU5NjY3OTY4NzUsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjU2LjQ0OTU2MjA3Mjc1MzkwNiwibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ4NzE0NDQ3MDIxNDg0NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIxMyJ9fV0%3D. Similarly, Hussainsagar in Hyderabad channels industrial effluents alongside domestic waste, fostering Planktothrix-dominated HABs that impair tourism and water supply (Sitote & Gebremedhine, 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllhYnNyYSBNZWxhayIsImZhbWlseSI6IlNpdG90ZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNaWNoYWVsIEdpcmltYXkiLCJmYW1pbHkiOiJHZWJyZW1lZGhpbmUifX1dLCJpZHMiOnsiaXNzbiI6W10sImlzYm4iOltdLCJkb2kiOiIxMC4yMDk0NC9wcmVwcmludHMyMDI0MDkuMDcwNC52MSIsIm9wZW5hbGV4IjoiVzQ0MDI0NjU0MTQiLCJleGEiOiJodHRwczovL3d3dy5wcmVwcmludHMub3JnL21hbnVzY3JpcHQvMjAyNDA5LjA3MDQifSwidGl0bGUiOiJDb21wcmVoZW5zaXZlIFJldmlldyBvZiBFdXRyb3BoaWNhdGlvbiBpbiBGcmVzaHdhdGVyIEVjb3N5c3RlbXM6IENhdXNlcywgRWZmZWN0cywgQXNzZXNzbWVudCwgYW5kIE1hbmFnZW1lbnQgU3RyYXRlZ2llcyIsImFic3RyYWN0IjoiVGhpcyByZXZpZXcgZXhhbWluZXMgZXV0cm9waGljYXRpb24gaW4gZnJlc2h3YXRlciBlY29zeXN0ZW1zLCBhIHdpZGVzcHJlYWQgZW52aXJvbm1lbnRhbCBpc3N1ZSBzdGVtbWluZyBmcm9tIGV4Y2Vzc2l2ZSBudXRyaWVudCBpbnB1dCwgbGFyZ2VseSBkdWUgdG8gaHVtYW4gYWN0aXZpdGllcy4gVGhlIHN0dWR5IGV4cGxvcmVzIHRoZSBvcmlnaW5zLCBjb25zZXF1ZW5jZXMsIGV2YWx1YXRpb24gdGVjaG5pcXVlcywgYW5kIGNvbnRyb2wgbWV0aG9kcyBmb3IgZXV0cm9waGljYXRpb24uIFByaW1hcnkgY2F1c2VzIGluY2x1ZGUgYWdyaWN1bHR1cmFsIGRpc2NoYXJnZSwgdXJiYW4gZGV2ZWxvcG1lbnQsIGFuZCBjbGltYXRlIHNoaWZ0cy4gVGhlIGVmZmVjdHMgYXJlIGZhci1yZWFjaGluZywgaW1wYWN0aW5nIGJpb2RpdmVyc2l0eSwgd2F0ZXIgcXVhbGl0eSwgYW5kIHNvY2lvLWVjb25vbWljIGZhY3RvcnMuIFRoZSBhc3Nlc3NtZW50IHV0aWxpemVzIGEgY29tYmluYXRpb24gb2YgY2hlbWljYWwsIHBoeXNpY2FsLCBhbmQgYmlvbG9naWNhbCBtYXJrZXJzLCB3aXRoIGN1dHRpbmctZWRnZSB0ZWNobm9sb2dpZXMgbGlrZSByZW1vdGUgc2Vuc2luZyBpbXByb3ZpbmcgbW9uaXRvcmluZyBlZmZvcnRzLiBNaXRpZ2F0aW9uIHN0cmF0ZWdpZXMgYXJlIGRpdmVyc2UsIGluY29ycG9yYXRpbmcgcHJldmVudGF0aXZlIGFjdGlvbnMgKHN1Y2ggYXMgbnV0cmllbnQgbGltaXRhdGlvbiBhbmQgbGFuZCBtYW5hZ2VtZW50KSwgY29ycmVjdGl2ZSBtZWFzdXJlcyAoaW5jbHVkaW5nIGJpby1tYW5pcHVsYXRpb24gYW5kIGNoZW1pY2FsIGFwcGxpY2F0aW9ucyksIGFuZCByZWd1bGF0b3J5IGluaXRpYXRpdmVzIChsaWtlIHdhdGVyIHF1YWxpdHkgZ3VpZGVsaW5lcyBhbmQgbnV0cmllbnQgZXhjaGFuZ2Ugc2NoZW1lcykuIFdoaWxlIGNhc2Ugc3R1ZGllcyBkZW1vbnN0cmF0ZSBzdWNjZXNzZnVsIGludGVydmVudGlvbnMsIHRoZXkgYWxzbyByZXZlYWwgb25nb2luZyBjaGFsbGVuZ2VzLCBwYXJ0aWN1bGFybHkgZWNvc3lzdGVtIHVucHJlZGljdGFiaWxpdHkgYW5kIGNsaW1hdGUgY2hhbmdlIGluZmx1ZW5jZXMuIFRoZSByZXNlYXJjaCBoaWdobGlnaHRzIHRoZSBwcm9taXNlIG9mIGVtZXJnaW5nIHRlY2hub2xvZ2llcywgbm90YWJseSBtYWNoaW5lIGxlYXJuaW5nLCBpbiBlbmhhbmNpbmcgbWFuYWdlbWVudCBwcmFjdGljZXMuIEtub3dsZWRnZSBnYXBzIHBlcnNpc3QsIGVzcGVjaWFsbHkgcmVnYXJkaW5nIGV1dHJvcGhpY2F0aW9uLWJpb2RpdmVyc2l0eSBkeW5hbWljcyBpbiB0cm9waWNhbCBlbnZpcm9ubWVudHMgYW5kIHRoZSBzb2Npby1lY29ub21pYyByYW1pZmljYXRpb25zIG9mIGludGVydmVudGlvbnMuIFRoZSBzdHVkeSBlbXBoYXNpemVzIHRoZSBpbXBvcnRhbmNlIG9mIGZsZXhpYmxlLCBjb21wcmVoZW5zaXZlIGFwcHJvYWNoZXMgdGhhdCBibGVuZCBzY2llbnRpZmljIGluc2lnaHRzIHdpdGggcHJhY3RpY2FsIHNvbHV0aW9ucyBhbmQgc3Ryb25nIHBvbGljeSBzdHJ1Y3R1cmVzIHRvIHRhY2tsZSB0aGlzIGludHJpY2F0ZSBlbnZpcm9ubWVudGFsIGNoYWxsZW5nZSBpbiBhbiBlcmEgb2YgY2xpbWF0ZSBjaGFuZ2UuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjIwOTQ0L3ByZXByaW50czIwMjQwOS4wNzA0LnYxIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjA5NDQvcHJlcHJpbnRzMjAyNDA5LjA3MDQudjEiLCJpc3N1ZWREYXRlIjoiMjAyNC0wOS0xMCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTEiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IlByZXByaW50cy5vcmciLCJpbXBhY3RGYWN0b3IiOiIxLjE1In0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjowLCJjaXRlZEJ5Q291bnQiOjR9fSwicXVvdGUiOiJUaGUgc3R1ZHkgZW1waGFzaXplcyB0aGUgaW1wb3J0YW5jZSBvZiBmbGV4aWJsZSwgY29tcHJlaGVuc2l2ZSBhcHByb2FjaGVzIHRoYXQgYmxlbmQgc2NpZW50aWZpYyBpbnNpZ2h0cyB3aXRoIHByYWN0aWNhbCBzb2x1dGlvbnMgYW5kIHN0cm9uZyBwb2xpY3kgc3RydWN0dXJlcyB0byB0YWNrbGUgdGhpcyBpbnRyaWNhdGUgZW52aXJvbm1lbnRhbCBjaGFsbGVuZ2UgaW4gYW4gZXJhIG9mIGNsaW1hdGUgY2hhbmdlLiBFdXRyb3BoaWNhdGlvbiwgY2F1c2VkIGJ5IGV4Y2Vzc2l2ZSBudXRyaWVudCBlbnJpY2htZW50IGluIHdhdGVyIGJvZGllcywgaXMgYSBzaWduaWZpY2FudCBlbnZpcm9ubWVudGFsIGNoYWxsZW5nZSBmb3IgZnJlc2h3YXRlciBlY29zeXN0ZW1zIG9uIGEgZ2xvYmFsIHNjYWxlLiBQcmltYXJpbHkgZHJpdmVuIGJ5IG5pdHJvZ2VuIGFuZCBwaG9zcGhvcnVzIGlucHV0cywgdGhpcyBwcm9jZXNzIHJlc3VsdHMgaW4gdGhlIG92ZXJwcm9kdWN0aW9uIG9mIGFsZ2FlIGFuZCBhcXVhdGljIHBsYW50cywgbGVhZGluZyB0byBhIGNhc2NhZGUgb2YgZWNvbG9naWNhbCBkaXN0dXJiYW5jZXMsIGluY2x1ZGluZyBveHlnZW4gZGVwbGV0aW9uIChoeXBveGlhKSwgbG9zcyBvZiBiaW9kaXZlcnNpdHksIGFuZCBzZXZlcmUgd2F0ZXIgcXVhbGl0eSBkZWdyYWRhdGlvbiAoS2FrYWRlIGV0IGFsLiwgMjAyMTsgTWVlcmhvZmYgZXQgYWwuLCAyMDIyOyBXdXJ0c2JhdWdoIGV0IGFsLiwgMjAxOSkuIn1d. Monsoon dilution offers transient relief, but legacy sediments recycle nutrients year-round, with particle-bound phosphorus resuspended during storms (Haxthausen, 2021)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkthcmwgUmFzbXVzIEF1Z3VzdCIsImZhbWlseSI6IkhheHRoYXVzZW4ifX1dLCJpZHMiOnsiaXNzbiI6W10sImlzYm4iOltdLCJtYWciOiIzMTc3MTEzMjA3Iiwib3BlbmFsZXgiOiJXMzE3NzExMzIwNyJ9LCJ0aXRsZSI6IlNlZGltZW50IG1pY3JvYmlhbCBmdWVsIGNlbGxzIGZvciBsYWtlIHJlc3RvcmF0aW9uIiwiYWJzdHJhY3QiOiJFdXRyb3BoaWNhdGlvbiBvZiBsYWtlcyBpcyB1YmlxdWl0b3VzLCBhcyAzMC00MCAlIG9mIHRoZSB3b3JsZOKAmXMgbGFrZXMgYXJlIGV1dHJvcGhpYywgYW5kIGNvbmRpdGlvbnMgY29udGludW91c2x5IGRldGVyaW9yYXRpbmcuIEV1dHJvcGhpY2F0aW9uIGlzIGEgY29uZGl0aW9uIG9mIG92ZXJmZXJ0aWxpemF0aW9uLCB3aGVyZSBhbGwgcmVxdWlyZW1lbnRzIGZvciByYXBpZCBncm93dGggb2YgYWxnYWUgYXJlIG1ldCwgcmVzdWx0aW5nIGluIGFsZ2FsIGJsb29tcy4gVGhpcyBjYXVzZXMgYSBzZWxmLXN1c3RhaW5lZCwgdmljaW91cyBjeWNsZSB3aGVyZSBwb29yIGVjb2xvZ2ljYWwgY29uZGl0aW9ucyBhcmUgbWFpbnRhaW5lZCB1bnRpbCBudXRyaWVudHMgYXJlIGxvc3QgZnJvbSB0aGUgbGFrZSBvdmVyIG1hbnkgeWVhcnMuIExha2UgcmVzdG9yYXRpb24gc2Vla3MgdG8gYnJlYWsgdGhpcyB2aWNpb3VzIGN5Y2xlLiBJbmFkZXF1YXRlIGF2YWlsYWJpbGl0eSBvZiBhbnkgc2luZ2xlIG51dHJpZW50IGluIHdpbGwgbGltaXQgYWxnYWwgZ3Jvd3RoLCB0eXBpY2FsbHkgcGhvc3Bob3J1cyBpcyB0YXJnZXRlZC4gVGhlIHNjb3BlIG9mIHRoaXMgUGhEIHRoZXNpcyB3YXMgdG8gZGV2ZWxvcCwgdGVzdCBhbmQgb3B0aW1pemUgc2VkaW1lbnQgbWljcm9iaWFsIGZ1ZWwgY2VsbHMgKFNNRkMpIGFzIGEgbm92ZWwgbWV0aG9kIGZvciBsYWtlIHJlc3RvcmF0aW9uLiBUaGUgdGhlc2lzIHNwYW5zIHRoZSB0d28gbWFpbiB0b3BpY3Mgb2YgU01GQ3MgYW5kIGxha2UgcmVzdG9yYXRpb24gdGVjaG5pcXVlcy4gTGFrZSByZXN0b3JhdGlvbiB0ZWNobmlxdWVzIGFyZSByZXZpZXdlZCwgcHJvdmlkaW5nIHVuZGVyc3RhbmRpbmcgb2YgdGhlIHVuZGVybHlpbmcgbWVjaGFuaXNtcy4gU01GQ3MgYXJlIGFwcHJvYWNoZWQgZnJvbSBhbiBlbmdpbmVlcmluZyBwZXJzcGVjdGl2ZSBhbmQgYSBmb2N1cyBvbiBsYWtlIHJlbWVkaWF0aW9uLiBUaGUgc3BlY2lmaWMgb2JqZWN0aXZlcyB3ZXJlIHRvIHRlc3QgYSBsb3ctY29zdCBlbGVjdHJvZGUgbWF0ZXJpYWwgZm9yIGl0cyBkdXJhYmlsaXR5LCBwcmFjdGljYWxpdHksIGFuZCBwZXJmb3JtYW5jZS4gUGVyZm9ybWFuY2Ugd2FzIGFzc2Vzc2VkIGFzIHRoZSBhYmlsaXR5IHRvIGxpbWl0IHBob3NwaG9ydXMgcmVsZWFzZSBmcm9tIHNlZGltZW50cywgaW5oaWJpdCBtZXRoYW5lIHJlbGVhc2UsIGFuZCB0aGUgYWRkZWQgYmVuZWZpdHMgb2YgZGVncmFkaW5nIHBvbGx1dGFudHMsIGFuZCBnZW5lcmF0aW5nIGVsZWN0cmljaXR5LiBQcm9vZiBvZiBjb25jZXB0IHdhcyBzaG93biBpbiBsYWJvcmF0b3J5IHJlYWN0b3JzICgxIEwpLCB0aGUgZWZmZWN0IG9mIHVwc2NhbGluZyB0byAyMDAgTCBpbiBuYXR1cmFsIGVudmlyb25tZW50IHdhcyBzdHVkaWVkIGluIG91dGRvb3IgbWVzb2Nvc21zLCBhbmQgdGhlbiB0ZXN0ZWQgYXQgcGlsb3Qgc2NhbGUgaW4gYSBldXRyb3BoaWMgZnJlc2h3YXRlciBsYWtlLiBQaWxvdCBpbnN0YWxsYXRpb25zIHdlcmUgdGhlIGxhcmdlc3QgaW4gdGhlIHdvcmxkIG9mIHRoZWlyIGtpbmQgYW5kIHdlcmUgb3BlcmF0ZWQgZm9yIDIgeWVhcnMuIFN0YWlubGVzcyBzdGVlbCB3YXMgc2VsZWN0ZWQgZm9yIHRlc3RpbmcgYXMgZWxlY3Ryb2RlIG1hdGVyaWFsIGR1ZSB0byBpdHMgbG93IGNvc3QsIGR1cmFiaWxpdHksIGFuZCBhdmFpbGFiaWxpdHkuIFBhaXJzIG9mIGVsZWN0cm9kZXMgd2VyZSBlbWJlZGRlZCBpbiB0aGUgc2VkaW1lbnQgKHRoaXMgaXMgY2FsbGVkIHRoZSBhbm9kZSkgYW5kIHN1c3BlbmRlZCBpbiB0aGUgb3Zlcmx5aW5nIHdhdGVyICh0aGlzIGlzIGNhbGxlZCB0aGUgY2F0aG9kZSkuIFRoZSB0d28gZWxlY3Ryb2RlcyB3ZXJlIGNvbm5lY3RlZCBhY3Jvc3MgYW4gZXh0ZXJuYWwgcmVzaXN0b3Igd2l0aCBlbGVjdHJpY2FsIHdpcmVzLiBNaWNyb2JlcyBpbiB0aGUgc2VkaW1lbnQgdXNlIHRoZSBhbm9kZSBwYWlyIGFzIGVsZWN0cm9uIGFjY2VwdG9yLCBlbmFibGluZyByZXNwaXJhdGlvbiBpbiB0aGUgYW5veGljIHNlZGltZW50LiBFbGVjdHJvbnMgdHJhdmVsIGZyb20gdGhlIHNlZGltZW50IHZpYSB0aGUgZWxlY3RyaWNhbCBjb25uZWN0b3IgdG8gdGhlIG92ZXJseWluZyBjYXRob2RlIHdoZXJlIG94eWdlbiBpbiB0aGUgd2F0ZXIgaXMgdGhlIHRlcm1pbmFsIGVsZWN0cm9uIGFjY2VwdG9yIGNvbXBsZXRpbmcgdGhlIGNpcmN1aXQuIEFzIGFuIGFkZGVkIGJlbmVmaXQsIGRlZ3JhZGF0aW9uIG9mIHBvbGx1dGFudHMgdXNpbmcgU01GQyB3YXMgdGVzdGVkIGluIG1hcmluZSBoYXJib3VyIHNlZGltZW50cywgYnV0IG5vIGNvbmNsdXNpdmUgcmVzdWx0cyBvYnRhaW5lZC4gTGFib3JhdG9yeSByZWFjdG9ycyB3ZXJlIG9wZXJhdGVkIGZvciBtb3JlIHRoYW4gMjAwIGRheXMgdXNpbmcgcGV0cm9sZXVtIGFzIHNvbGUgY2FyYm9uIHNvdXJjZSwgY29uZmlybWluZyBvaWwgZGVncmFkYXRpb24gYnkgU01GQ3MuIEEgcmVtb3ZhbCByYXRlIG9mIDAuOSBnIHBldHJvbGV1bSBtLTMgZC0xIHJlYWN0b3Igdm9sdW1lIHdhcyBlc3RpbWF0ZWQgZnJvbSBtZWFzdXJlZCBlbGVjdHJpY2FsIG91dHB1dC4gVGhpcyB3YXMgZG9uZSBieSBhc3N1bXB0aW9uIG9mIHRoZSBjaGVtaWNhbCByZWFjdGlvbiBvY2N1cnJpbmcsIGZyb20gdGhpcyB0aGUgbnVtYmVyIG9mIGVsZWN0cm9ucyB0cmFuc2ZlcnJlZCBmcm9tIG9pbCB0byB3YXRlciB3YXMga25vd24gYW5kIGNvdWxkIGJlIGNvcnJlbGF0ZWQgdG8gdGhlIG1lYXN1cmVkIGZsb3cgb2YgZWxlY3Ryb25zIChjdXJyZW50KSB0aHJvdWdoIHRoZSBleHRlcm5hbCBjaXJjdWl0LiBTdGFpbmxlc3Mgc3RlZWwgZWxlY3Ryb2RlcyB3ZXJlIG9wZXJhdGVkIGluIGZyZXNod2F0ZXIgbGFrZSBlbnZpcm9ubWVudHMgZm9yIG1vcmUgdGhhbiA0IHllYXJzIHdpdGggbWluaW1hbCBkYW1hZ2UgdG8gdGhlIG1hdGVyaWFsLiBUaGUgZWxlY3Ryb2RlIGlzIHRoZSBtYWluIG1hdGVyaWFsIGNvc3QgaW4gc3VjaCBzeXN0ZW1zLiBIb3dldmVyLCBBSVNJMzE2IHN0YWlubGVzcyBzdGVlbCBzaW1pbGFyIHRvIHdoYXQgd2FzIHVzZWQgaW4gdGhpcyB0aGVzaXMgd2FzIGF0IHRoZSB0aW1lIG9mIHdyaXRpbmcgYXZhaWxhYmxlIGF0IGJ1bGsgY29zdCBvZiAwLjA5MSDigqwgbS0yLCBjb3JyZXNwb25kaW5nIHRvIDkxMCDigqwgYWNyZS0xLCBzaWduaWZpY2FudGx5IGxvd2VyIHRoYW4gb3RoZXIgY29tbW9uIG1hdGVyaWFscy4gQ2F0aG9kZXMgYXQgbGFyZ2Ugc2NhbGUgY291bGQgYmUgbW92ZWQgaG9yaXpvbnRhbGx5IGFuZCB2ZXJ0aWNhbGx5IHdpdGggbWlub3IgaW1wYWN0IHRvIHBlcmZvcm1hbmNlLiBQaWxvdCBzY2FsZSBTTUZDcyBnZW5lcmF0ZWQgMC4zODMgbVcgbS0yIHN1c3RhaW5lZCBhbmQgMjcuNCBtVyBtLTIgcGVhayBvdXRwdXQuIFRoZSBzeXN0ZW0gd2FzIGxpbWl0ZWQgYnkgdGhlIGNhdGhvZGUgc3VyZmFjZSBhcmVhLCBtZWFuaW5nIG11bHRpLWxheWVyIGNhdGhvZGVzIHNob3VsZCBiZSB0ZXN0ZWQgaW4gdGhlIGZ1dHVyZSBpZiBlbGVjdHJpY2FsIHBvd2VyIHByb2R1Y3Rpb24gaXMgdGhlIGdvYWwuIFRoZSBjYXRob2RlIHNob3VsZCBiZSBwbGFjZWQgYXQgYSBkZXB0aCB0aGF0IGVuc3VyZXMgaGlnaCBkaXNzb2x2ZWQgb3h5Z2VuIGNvbmNlbnRyYXRpb25zIHRvIGF2b2lkIHRoZSBzeXN0ZW0gdG8gYmUgbGltaXRlZC4gSG93ZXZlciwgdG9vIGhpZ2ggZWxlY3RyaWNpdHkgZ2VuZXJhdGlvbiB3YXMgZm91bmQgY291bnRlcnByb2R1Y3RpdmUgZm9yIHJlbWVkaWF0aW9uIHB1cnBvc2VzIGJlY2F1c2UgdGhpcyB3b3VsZCBpbmNyZWFzZSBzZWRpbWVudCB0dXJub3ZlciByYXRlLCByZWxlYXNpbmcgcG9sbHV0YW50cyBzdG9yZWQgaW4gdGhlIHNlZGltZW50cy4gUCBiaW5kaW5nIHdhcyBub3QgZm91bmQgdG8gcmVxdWlyZSBoaWdoIHR1cm5vdmVyIG9mIHNlZGltZW50IG9yZ2FuaWMgbWF0dGVyLiBUaGUgdHVybm92ZXIgY2FuIGJlIGNvbnRyb2xsZWQgYnkgY2hhbmdpbmcgdGhlIGV4dGVybmFsIHJlc2lzdGFuY2UuIEhpZ2ggcmVzaXN0YW5jZSBsaW1pdHMgdGhlIGVsZWN0cmljYWwgcGVyZm9ybWFuY2UsIGxvdyByZXNpc3RhbmNlIGluY3JlYXNlcyBzZWRpbWVudCB0dXJub3ZlciBhbmQgcmlza3MgZGVzdGFiaWxpemluZyB0aGUgc3lzdGVtLiBUbyBhY2hpZXZlIHJlZHVjZWQgY29uY2VudHJhdGlvbnMgb2YgcGhvc3BoYXRlIGluIHRoZSB3YXRlciBjb2x1bW4sIGFub2RlcyBtdXN0IGJlIHBsYWNlZCBpbiB0aGUgc2VkaW1lbnQgc3VyZmFjZSBzaW5jZSBhIGZldyBtaWxsaW1ldHJlcyBvZiBzZWRpbWVudCBtYXkgY29udGFpbiBhbmQgcmVsZWFzZSBlbm91Z2ggUCB0byBzdXN0YWluIGV1dHJvcGhpY2F0aW9uLiBQaG9zcGhhdGUgd2FzIG1lYXN1cmVkIGluIHRoZSB3YXRlciBjb2x1bW4gbmVhciB0aGUgc2VkaW1lbnQsIGluIHN5c3RlbXMgd2l0aCBhbmQgd2l0aG91dCBlbGVjdHJvZGVzLiBQaG9zcGhhdGUgY29uY2VudHJhdGlvbnMgd2VyZSBzdWNjZXNzZnVsbHkgcmVkdWNlZCBieSB1cCB0byA5NSAlIGluIDItY2hhbWJlciBsYWJvcmF0b3J5IHJlYWN0b3JzLCBhbmQgYnkgdXAgdG8gOTggJSBpbiB0aGUgcGlsb3QgaW5zdGFsbGF0aW9uLiBJbiB0aGUgcGlsb3QgZXhwZXJpbWVudCwgcGhvc3BoYXRlIGNvbmNlbnRyYXRpb25zIGFib3ZlIGVsZWN0cm9kZXMgd2VyZSBsb3dlciBpbiBhbGwgbWVhc3VyZW1lbnRzLCBjb21wYXJlZCB0byBjb250cm9scy4gU2VkaW1lbnQgcmVkb3ggcG90ZW50aWFscyBpbmNyZWFzZWQgZnJvbSAtNDAwIG1WIHZzIHN0YW5kYXJkIGh5ZHJvZ2VuIGVsZWN0cm9kZSAoU0hFKSB0byAtMTc1IG1WIHZzIFNIRSwgY29ycmVzcG9uZGluZyB0byBhIG1vdmUgZnJvbSBhIG1ldGhhbm9nZW5ldGljIGRvbWFpbiB0byBzdWxwaHVyIHJlZHVjaW5nLiBBbmFseXNpcyBvZiB0aGUgbWljcm9iaWFsIGNvbW11bml0eSBpbmRpY2F0ZWQgZW5oYW5jZWQgc3VscGh1ciBjeWNsaW5nIHdoZXJlIGVsZWN0cm9kZXMgb3hpZGl6ZSBzdWxwaGlkZSB3aGljaCB0aGUgYmFjdGVyaWEgYWdhaW4gcmVkdWNlIHRvIG9idGFpbiBlbmVyZ3kuIEl0IHdhcyBoeXBvdGhlc2lzZWQgdGhhdCBTTUZDIHByb21vdGUgaGlnaGVyIHJlZG94IHBvdGVudGlhbHMgYnkgZmFjaWxpdGF0aW5nIG94aWRhdGlvbiBvZiBzdWxwaHVyIGFuZCBpcm9uIHNwZWNpZXMsIGluaGliaXRpbmcgSDJTIGZvcm1hdGlvbiBhbmQgRmUgc2NhdmVuZ2luZyBieSBGZVMgZm9ybWF0aW9uLCB0aHVzIGFsbG93aW5nIGZvciBQIGJpbmRpbmcuIEEgbG93ZXIgcmVsYXRpdmUgYWJ1bmRhbmNlIG9mIG1ldGhhbm9nZW5zIG9uIGVsZWN0cm9kZXMgY29tcGFyZWQgdG8gY29udHJvbHMgc3VnZ2VzdHMgc3VwcHJlc3Npb24gb3Igb3V0Y29tcGV0aW5nIG9mIG1ldGhhbm9nZW5zLlF1YW50aWZpY2F0aW9uIG9mIG1ldGhhbmUgcHJvZHVjdGlvbiB3YXMgZG9uZSBvbiB0aGUgYmFzaXMgb2YgYSBDSDQ6Q08yIHJhdGlvIHRvIGFsbG93IGNvbXBhcmFiaWxpdHkgYmV0d2VlbiBzeXN0ZW1zLiBNZXRoYW5lIGVtaXNzaW9uIHdhcyByZWR1Y2VkIGJ5IHVwIHRvIDk5ICUgYW5kIG1haW50YWluZWQgaW5oaWJpdGlvbiBmb3Igc2V2ZXJhbCBtb250aHMgaW4gbGFib3JhdG9yeSBzeXN0ZW1zIHdpdGggZWxlY3Ryb2Rlcy4gSW4gdGhlIHBpbG90IGluc3RhbGxhdGlvbnMgbm8gZ2FzIHNhbXBsZXMgY291bGQgYmUgY29sbGVjdGVkLCBhbmQgbm8gQ08yOkNINCByYXRpb3Mgd2VyZSBvYnRhaW5lZC4gQWVyYXRpb24gaXMgdGhlIGN1cnJlbnQgcmVzdG9yYXRpb24gdGVjaG5pcXVlIHRoYXQgbW9zdCBjbG9zZWx5IHJlc2VtYmxlcyB0aGUgcHJvcG9zZWQgU01GQyB0ZWNobmlxdWUuIEFlcmF0aW9uIGhhcyBwcmV2aW91c2x5IGZhaWxlZCB0byByZWR1Y2UgUCByZWxlYXNlIGluIHNldmVyYWwgc2VwYXJhdGUgaW52ZXN0aWdhdGlvbnMuIFNNRkNzIGFzIGEgcmVtZWRpYXRpb24gdG9vbCB3YXMgZm91bmQgdG8gb2ZmZXIgc2V2ZXJhbCBiZW5lZml0cyBvdmVyIGNvbnZlbnRpb25hbCBtZXRob2RzOiBTTUZDcyBvZmZlcnMgY29udGludW91cyBlZmZlY3QsIHdoaWxlIG90aGVyIHRlY2huaXF1ZXMgcmVxdWlyZSBjb250aW51b3VzIG9yIHJlcGVhdGVkIHRyZWF0bWVudC4gU01GQ3MgY2FuIGJlIGNvbnRyb2xsZWQgaW4gcmVhbC10aW1lIGJ5IGFsdGVyaW5nIHRoZSBleHRlcm5hbCByZXNpc3RvciBhbmQgY2FuIGJlIGVhc2lseSBtb25pdG9yZWQgYnkgbWVhc3VyaW5nIHRoZSBwcm9kdWNlZCBlbGVjdHJpY2FsIG91dHB1dC4gSWYgaW1wbGVtZW50ZWQgYXQgbGFyZ2Ugc2NhbGUgaXQgaXMgZXhwZWN0ZWQgdGhhdCB0aGUgZWxlY3Ryb2RlIGZpZWxkcyB3aWxsIGhlbHAgbWFpbnRhaW4gbG93IHBob3NwaGF0ZSBjb25jZW50cmF0aW9ucyBpbiB0aGUgd2F0ZXIgZm9yIGRlY2FkZXMgd2l0aCBtaW5pbWFsIGludGVydmVudGlvbiwgYW5kIGFmdGVyIHNvbWUgdGltZSwgYmVudGhpYyBwbGFudHMgYXJlIGV4cGVjdGVkIHRvIHJldHVybiwgdGh1cyBtYWludGFpbmluZyB0aGUgYmFsYW5jZSBoZW5jZSBmb3J0aC4iLCJ1cmwiOiJodHRwczovL2xvY2FsLmZvcnNrbmluZ3Nwb3J0YWwuZGsvbG9jYWwvZGtpLWNnaS93cy9jcmlzLWxpbms%2Fc3JjPWR0dSZpZD1kdHUtZDY2ZDMzNzAtMjU2Zi00ODg1LWEwOWUtMzM2YTkyNGY0YmVkJnRpPVNlZGltZW50JTIwbWljcm9iaWFsJTIwZnVlbCUyMGNlbGxzJTIwZm9yJTIwbGFrZSUyMHJlc3RvcmF0aW9uIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2JhY2tlbmQub3JiaXQuZHR1LmRrL3dzL2ZpbGVzLzI0ODA2OTU4OS9UaGVzaXNfb25saW5lX3ZlcnNpb25fS2FybF9IYXh0aGF1c2VuLnBkZiIsImlzc3VlZERhdGUiOiIyMDIxLTAxLTAxIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0wOCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiUmVzZWFyY2ggUG9ydGFsIERlbm1hcmsiLCJpbXBhY3RGYWN0b3IiOiIwLjA0In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6NzZ9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiVGVjaG5pY2FsIFVuaXZlcnNpdHkgb2YgRGVubWFyayJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MCwiY2l0ZWRCeUNvdW50IjoxfX0sInF1b3RlIjoiRXh0ZXJuYWwgaW5wdXRzIGluY3JlYXNlIFAgY29uY2VudHJhdGlvbiBpbiAgdGhlIHdhdGVyLCBjYXVzaW5nIGFsZ2FsIGJsb29tcyB0aGF0IHNoYWRvdyB0aGUgbGFrZSBib3R0b20sIGFuZCBiZW50aGljIG1hYy1yb3BoeXRlcyByZWNlZGUuIFByZWRhdG9yeSBmaXNoIGFyZSB1bmFibGUgdG8gaHVudCwgYW5kIHRoZWlyIGZvb2Qgc291cmNlIGlzICBkaXNydXB0ZWQgc3VjaCBhcyBmaXNoIGZlZWRpbmcgb24gYmVudGhpYyBpbnZlcnRlYnJhdGVzIGFuZCBib3R0b20tZHdlbGxpbmcgYW5pbWFscyBhcmUgc3VmZm9jYXRlZCAoVmFuZGVyIFphbmRlbiBhbmQgVmFkZWJvbmNvZXVyIDIwMDIpLCBzbyB0aGF0IHBsYW5rdGl2b3JvdXMgZmlzaCBzcGVjaWVzIGRvbWluYXRlLiBUaGlzIGluY3JlYXNlcyBwcmVkYXRpb24gb24gem9vIHBsYW5rdG9uICBhbmQgcmVkdWNlcyBncmF6aW5nIG9uIGFsZ2FlLiBBbGdhZSBwcm9saWZlcmF0ZSwgZGllLCBhbmQgc2luayB0byB0aGUgYm90dG9tLCAgY29uc3VtaW5nIG94eWdlbiB3aGVuIGRlZ3JhZGVkLCB3aGljaCBsZWFkcyB0byBveHlnZW4gZGVmaWNpdCBpbiB0aGUgYm90LXRvbSwga2lsbGluZyBmaXNoIChXdXJ0c2JhdWdoIGV0IGFsLiAyMDE5KS4gRmlndXJlIDI6IFZpY2lvdXMgY3ljbGVzIG9mIGV1dHJvcGhpY2F0aW9uLiBMZWZ0OiBJbnRlcmxpbmtlZCBwcm9jZXNzZXMgc3VzdGFpbmluZyBldXRyb3BoaS1jYXRpb24gaW4gYSBsYWtlLiBCbHVlOiBwcm9jZXNzZXMgaW4gd2F0ZXIsIGdyZWVuOiBwcm9jZXNzZXMgaW52b2x2aW5nIG1pY3JvYWxnYWUsIGdyZXk6IHNlZC1pbWVudCByZWxhdGVkIHByb2Nlc3NlcywgeWVsbG93OiBwcm9jZXNzZXMgaW52b2x2aW5nIGZhdW5hLiBSaWdodCwgZGFzaGVkIGxpbmU6IFRoZSBvcmRlciBpbiB3aGljaCBlbGVjdHJvbiBhY2NlcHRvcnMgYXJlIGNvbnN1bWVkIGluIHRoZSBhYnNlbmNlIG9mIE8yIHNob3duIGFzIGNvbmNlbnRyYXRpb24gdmVyc3VzICBkZXB0aCBpbiBzZWRpbWVudCwgYW5kIGF0IHdoaWNoIHBvdGVudGlhbCB0aGlzIG9jY3VycyAoQmVybmVyIDE5ODEsIGJ5IGNvdXJ0ZXN5IG9mIHRoZSBzb2NpZXR5ICBmb3Igc2VkaW1lbnRhcnkgZ2VvbG9neSkuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxNywiYmJveCI6eyJ0b3AiOjc5LjkzMzE1MTI0NTExNzE5LCJsZWZ0IjoxNC4yOTU1MDM2MTYzMzMwMDgsIndpZHRoIjo3MS45Mzg5MzQzMjYxNzE4OCwiaGVpZ2h0IjoyLjA1MzQ3NzI4NzI5MjQ4MDV9fSx7InBhZ2VOdW1iZXIiOjE3LCJiYm94Ijp7InRvcCI6ODEuOTcxMzQzOTk0MTQwNjIsImxlZnQiOjE0LjI5NTUwMzYxNjMzMzAwOCwid2lkdGgiOjcxLjMzMzAwMDE4MzEwNTQ3LCJoZWlnaHQiOjIuMDUzNDc3Mjg3MjkyNDgwNX19LHsicGFnZU51bWJlciI6MTcsImJib3giOnsidG9wIjo4NC4wMjM3OTYwODE1NDI5NywibGVmdCI6MTQuMjk1NTAzNjE2MzMzMDA4LCJ3aWR0aCI6NzEuODEyMDE5MzQ4MTQ0NTMsImhlaWdodCI6Mi4wNTM0NzcyODcyOTI0ODA1fX0seyJwYWdlTnVtYmVyIjoxNywiYmJveCI6eyJ0b3AiOjg2LjA2MTk5NjQ1OTk2MDk0LCJsZWZ0IjoxNC4yOTU1MDM2MTYzMzMwMDgsIndpZHRoIjo3MS45NTczNTkzMTM5NjQ4NCwiaGVpZ2h0IjoyLjA1MzQ3NzI4NzI5MjQ4MDV9fSx7InBhZ2VOdW1iZXIiOjE4LCJiYm94Ijp7InRvcCI6OS45MjY0ODc5MjI2Njg0NTcsImxlZnQiOjE0LjI5NTUwMzYxNjMzMzAwOCwid2lkdGgiOjcxLjk3NDA2MDA1ODU5Mzc1LCJoZWlnaHQiOjEwLjIzNDc3NzQ1MDU2MTUyM319LHsicGFnZU51bWJlciI6MTgsImJib3giOnsidG9wIjo0NC43MzQwNDMxMjEzMzc4OSwibGVmdCI6MTQuMjk1NTAzNjE2MzMzMDA4LCJ3aWR0aCI6NzEuODkzNDMyNjE3MTg3NSwiaGVpZ2h0IjoxMC40MDMyODk3OTQ5MjE4NzV9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI5MyJ9fV0%3D. These dynamics shift lakes from N- to P-limitation seasonally, favoring diazotrophs and toxin-producers interchangeably (Nõges et al., 2020; Swann et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1pY2FoIE0uIiwiZmFtaWx5IjoiU3dhbm4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQWxpY2lhIiwiZmFtaWx5IjoiQ29ydMOpcyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBbGV4YW5kZXIgTC4iLCJmYW1pbHkiOiJGb3JyZXN0In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5pY2hvbGFzIFQuIiwiZmFtaWx5IjoiRnJhbXN0ZWQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU3RldmVuIiwiZmFtaWx5IjoiU2Fkcm8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiR2VvZmZyZXkiLCJmYW1pbHkiOiJTY2hsYWRvdyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBbmdlbGEgRGUiLCJmYW1pbHkiOiJQYWxtYeKAkERvdyJ9fV0sImlkcyI6eyJpc3NuIjpbIjEwMTUtMTYyMSIsIjE0MjAtOTA1NSJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMTAwNy9zMDAwMjctMDI0LTAxMDQ1LTIiLCJvcGVuYWxleCI6Ilc0MzkyNzEzMzk5IiwiZXhhIjoiaHR0cHM6Ly9saW5rLnNwcmluZ2VyLmNvbS9hcnRpY2xlLzEwLjEwMDcvczAwMDI3LTAyNC0wMTA0NS0yP2Vycm9yPWNvb2tpZXNfbm90X3N1cHBvcnRlZCZjb2RlPTgzY2Y4ZDg4LWM5YTAtNGNkZC04MjU2LTFhODVlNmFiZGFjMyJ9LCJ0aXRsZSI6IkludGVybmFsIHBob3NwaG9ydXMgbG9hZGluZyBhbHRlcnMgbnV0cmllbnQgbGltaXRhdGlvbiBhbmQgY29udHJpYnV0ZXMgdG8gY3lhbm9iYWN0ZXJpYWwgYmxvb21zIGluIGEgcG9seW1pY3RpYyBsYWtlIiwiYWJzdHJhY3QiOiJBYnN0cmFjdCBDbGVhciBMYWtlLCBhIG1lZGl1bS1zaXplZCBoeXBlcmV1dHJvcGhpYywgcG9seW1pY3RpYyBsYWtlIGluIG5vcnRoZXJuIENhbGlmb3JuaWEsIGhhcyBoYWQgcmVjdXJyaW5nIGhhcm1mdWwgY3lhbm9iYWN0ZXJpYSBibG9vbXMgKEhDQnMpIGZvciBvdmVyIGEgY2VudHVyeSBkZXNwaXRlIHJlZHVjdGlvbnMgaW4gZXh0ZXJuYWwgcGhvc3Bob3J1cyAoUCkgbG9hZGluZ3MuIEludGVybmFsIFAgbG9hZGluZyBjYW4gYWx0ZXIgbnV0cmllbnQgYXZhaWxhYmlsaXR5IGFuZCBsaW1pdGF0aW9uIHN1cHBvcnRpbmcgSENCcyBidXQgaXMgcmFyZWx5IHF1YW50aWZpZWQgb3IgY29tcGFyZWQgd2l0aCBleHRlcm5hbCBsb2Fkcy4gV2UgaGF2ZSBxdWFudGlmaWVkIGV4dGVybmFsIFAgbG9hZHMgZnJvbSAyMDE5IHRvIDIwMjEgZm9yIHRoZSB0aHJlZSBtYWluIHRyaWJ1dGFyaWVzIChhY2NvdW50aW5nIGZvciA0NiUgb2YgdGhlIGZsb3cpIGFuZCBpbnRlcm5hbCBsb2FkaW5ncyB1c2luZyB0d28gbWV0aG9kczogYSBQIG1hc3MgYmFsYW5jZSBhbmQgbW9kZWxlZCByZWxlYXNlIHJhdGVzIG9mIHNvbHVibGUgcmVhY3RpdmUgcGhvc3Bob3J1cyBmcm9tIG94aWMgYW5kIGFub3hpYyBzZWRpbWVudHMuIEluIGFkZGl0aW9uLCB3ZSBjb21iaW5lZCBoaWdoLWZyZXF1ZW5jeSBpbiBzaXR1IG1lYXN1cmVtZW50cyBvZiB3YXRlciB0ZW1wZXJhdHVyZSBhbmQgZGlzc29sdmVkIG94eWdlbiwgZGlzY3JldGUgZ3JhYiBzYW1wbGluZyBmb3IgbnV0cmllbnQgY2hlbWlzdHJ5LCBhbmQgcmVtb3RlIHNlbnNpbmcgdG8gZXhwbG9yZSB0aGUgcG90ZW50aWFsIGRyaXZlcnMgb2YgdGhlIG9ic2VydmVkIHZhcmlhYmlsaXR5IGFuZCBwcm92aWRlIGEgY29tcHJlaGVuc2l2ZSB2aWV3IG9mIHRoZSBzcGF0aW90ZW1wb3JhbCBkeW5hbWljcyBvZiBIQ0JzLiBCeSB1bmRlcnN0YW5kaW5nIHRoZSByZWxhdGl2ZSBjb250cmlidXRpb24gb2YgZXh0ZXJuYWwgYW5kIGludGVybmFsIG51dHJpZW50IGxvYWRpbmdzIGFuZCB0aGUgcmVsYXRpb25zaGlwIGJldHdlZW4gZW52aXJvbm1lbnRhbCBwYXJhbWV0ZXJzIGFuZCBIQ0JzLCBpbnRlcmFubnVhbCBibG9vbSB2YXJpYWJpbGl0eSBjYW4gYmUgYmV0dGVyIHByZWRpY3RlZC4gQ29tcGFyYXRpdmUgZXN0aW1hdGVzIG9mIGV4dGVybmFsIGFuZCBpbnRlcm5hbCBwaG9zcGhvcnVzIGxvYWRpbmcgaW5kaWNhdGUgdGhhdCBpbnRlcm5hbCBzb3VyY2VzIGFjY291bnRlZCBmb3IgNzDigJM5NSUgb2YgdGhlIHRvdGFsIFAgaW5wdXQgaW50byB0aGUgc3lzdGVtIGR1cmluZyB0aGUgc3R1ZHkgcGVyaW9kLiBDb250cmFyeSB0byBvdGhlciBsYWtlcywgdGhlIGludGVuc2l0eSBvZiB0aGUgc3VtbWVyIGJsb29tIHNlYXNvbiB3YXMgY29ycmVsYXRlZCB0byB0aGUgdGltaW5nIGFuZCBkdXJhdGlvbiBvZiBhbm94aWEgcmF0aGVyIHRoYW4gdGhlIG1hZ25pdHVkZSBvZiBzcHJpbmcgcnVub2ZmLiBJbnRlcm5hbGx5IHJlbGVhc2VkIFAgc2hpZnRlZCB0aGUgc3lzdGVtIGZyb20gcGhvc3Bob3J1cyB0byBuaXRyb2dlbiBsaW1pdGF0aW9uIGR1cmluZyB0aGUgc3VtbWVyLCBwb3RlbnRpYWxseSBmYXZvcmluZyB0aGUgcHJvbGlmZXJhdGlvbiBvZiBuaXRyb2dlbi1maXhpbmcgY3lhbm9iYWN0ZXJpYS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAwNy9zMDAwMjctMDI0LTAxMDQ1LTIiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vbGluay5zcHJpbmdlci5jb20vY29udGVudC9wZGYvMTAuMTAwNy9zMDAwMjctMDI0LTAxMDQ1LTIucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjQtMDMtMTIiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTExIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJBcXVhdGljIFNjaWVuY2VzIiwiaXNzdWUiOjIsInZvbHVtZSI6ODYsImltcGFjdEZhY3RvciI6IjIuODQifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiU3ByaW5nZXIgU2NpZW5jZStCdXNpbmVzcyBNZWRpYSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NzUsImNpdGVkQnlDb3VudCI6MjF9fSwicXVvdGUiOiJDb250cmFyeSB0byBvdGhlciBsYWtlcywgdGhlIGludGVuc2l0eSBvZiB0aGUgc3VtbWVyIGJsb29tIHNlYXNvbiB3YXMgY29ycmVsYXRlZCB0byB0aGUgdGltaW5nIGFuZCBkdXJhdGlvbiBvZiBhbm94aWEgcmF0aGVyIHRoYW4gdGhlIG1hZ25pdHVkZSBvZiBzcHJpbmcgcnVub2ZmLiBJbnRlcm5hbGx5IHJlbGVhc2VkIFAgc2hpZnRlZCB0aGUgc3lzdGVtIGZyb20gcGhvc3Bob3J1cyB0byBuaXRyb2dlbiBsaW1pdGF0aW9uIGR1cmluZyB0aGUgc3VtbWVyLCBwb3RlbnRpYWxseSBmYXZvcmluZyB0aGUgcHJvbGlmZXJhdGlvbiBvZiBuaXRyb2dlbi1maXhpbmcgY3lhbm9iYWN0ZXJpYS4gLSBbQ2FsY2l1bSBhbmQgcGhvc3BoYXRlIG1ldGFib2xpYyBkaXNvcmRlcnNdKGh0dHBzOi8vbGluay5zcHJpbmdlci5jb20vc3ViamVjdHMvY2FsY2l1bS1hbmQtcGhvc3BoYXRlLW1ldGFib2xpYy1kaXNvcmRlcnMpIEV1dHJvcGhpY2F0aW9uIHBvc2VzIGEgZ3JhdmUgdGhyZWF0IHRvIGxlbnRpYyBlY29zeXN0ZW1zIGdsb2JhbGx5LCBwcm9tb3RpbmcgdGhlIHByb2xpZmVyYXRpb24gb2YgaGFybWZ1bCBjeWFub2JhY3RlcmlhIGJsb29tcyAoSENCcykgdGhhdCByZWR1Y2UgYXF1YXRpYyBiaW9kaXZlcnNpdHksIHlpZWxkaW5nIGluY3JlYXNlZCB3YXRlciB0cmVhdG1lbnQgY29zdHMsIGFuZCB0aHJlYXRlbmluZyBodW1hbiBoZWFsdGggdGhyb3VnaCBibG9vbS1wcm9kdWNlZCBjeWFub3RveGlucyAoQXLDoW96IGV0IGFsLiJ9LHsib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRpaW5hIiwiZmFtaWx5IjoiTsO1Z2VzIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hc2ltZSIsImZhbWlseSI6IkphbmF0aWFuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlJlZXQiLCJmYW1pbHkiOiJMYXVnYXN0ZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQZWV0ZXIiLCJmYW1pbHkiOiJOw7VnZXMifX1dLCJpZHMiOnsiaXNzbiI6WyIyMzUxLTk4OTQiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMTYvai5nZWNjby4yMDIwLmUwMTM2OSIsIm1hZyI6IjMxMDU0NDYwOTAiLCJvcGVuYWxleCI6IlczMTA1NDQ2MDkwIiwiZXhhIjoiaHR0cHM6Ly93d3cuc2NpZW5jZWRpcmVjdC5jb20vc2NpZW5jZS9hcnRpY2xlL3BpaS9TMjM1MTk4OTQyMDMwOTEwMCJ9LCJ0aXRsZSI6IlBvc3Qtc292aWV0IGNoYW5nZXMgaW4gbml0cm9nZW4gYW5kIHBob3NwaG9ydXMgc3RvaWNoaW9tZXRyeSBpbiB0d28gbGFyZ2Ugbm9uLXN0cmF0aWZpZWQgbGFrZXMgYW5kIHRoZSBpbXBhY3Qgb24gcGh5dG9wbGFua3RvbiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2ouZ2VjY28uMjAyMC5lMDEzNjkiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2ouZ2VjY28uMjAyMC5lMDEzNjkiLCJpc3N1ZWREYXRlIjoiMjAyMC0xMS0yMCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTEiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6Ikdsb2JhbCBFY29sb2d5IGFuZCBDb25zZXJ2YXRpb24iLCJ2b2x1bWUiOjI0LCJpbXBhY3RGYWN0b3IiOiIzLjU5In0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkVsc2V2aWVyIEJWIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo3MSwiY2l0ZWRCeUNvdW50IjoxNH19LCJxdW90ZSI6IlJhbmRvbSBGb3Jlc3QgYW5hbHlzaXMgaW5kaWNhdGVkIGFuIGltcG9ydGFudCByb2xlIG9mIGxpZ2h0IGxpbWl0YXRpb24gaW4gYm90aCBsYWtlcy4gQ29uc3RhbnRseSBsb3dlciBsZXZlbHMgb2YgTiBpbiB0aGUgZGVlcGVyIExha2UgUGVpcHNpIGZhdm91cmVkIE4yLWZpeGluZyBzcGVjaWVzLCB3aGljaCwgYXMgYSBwYXJhZG94LCBiZWNhbWUgUC1saW1pdGVkLiBBcyBjbGltYXRlIHdhcm1pbmcgcmVpbmZvcmNlcyBldXRyb3BoaWNhdGlvbiBwaGVub21lbmEgaW4gbGFrZXMgYnkgaW5jcmVhc2luZyBpbnRlcm5hbCBudXRyaWVudCBsb2FkaW5nIGFuZCBmYXZvdXJpbmcgYmxvb20tZm9ybWluZyBjeWFub2JhY3RlcmlhLCBtb3JlIHN0cmluZ2VudCBtZWFzdXJlcyB3b3VsZCBiZSBuZWVkZWQgdG8gZnVydGhlciBsaW1pdCBudXRyaWVudCBsb2FkcyAoZXNwZWNpYWxseSB0aGF0IG9mIFApIHRvIGxha2VzIHRocm91Z2ggaW1wcm92ZWQgd2FzdGV3YXRlciB0cmVhdG1lbnQgYW5kIGluY3JlYXNlZCBlZmZpY2llbmN5IG9mIGZlcnRpbGlzZXIgYXBwbGljYXRpb24uIExvbmctdGVybSBjaGFuZ2VzIGluIG51dHJpZW50IGxvYWRpbmcgYW5kIHN0b2ljaGlvbWV0cnkgaW4gd2F0ZXIgYm9kaWVzIHJlZmxlY3QgY2hhbmdlcyBpbiBhZ3JpY3VsdHVyYWwgcHJhY3RpY2VzIGFuZCB3YXN0ZXdhdGVyIHRyZWF0bWVudC4ifV0%3D. Public health risks escalate as untreated water contaminates groundwater and informal supply chains, with cyanotoxins linked to livestock deaths and human illnesses (Gobler et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkNocmlzdG9waGVyIEouIiwiZmFtaWx5IjoiR29ibGVyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlJ1dGggVy4iLCJmYW1pbHkiOiJEcmlua3dhdGVyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkEuIFcuIiwiZmFtaWx5IjoiQW50aG9ueSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJKZW5uaWZlciBBLiIsImZhbWlseSI6IkdvbGVza2kifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW5uIE1hcmllIEUuIiwiZmFtaWx5IjoiRmFtdWxhcm8tUGVjb3JhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1pY2hhZWwgQi4iLCJmYW1pbHkiOiJXYWxsYWNlIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5vcmEgUi4gVy4iLCJmYW1pbHkiOiJTdHJhcXVhZGluZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJSb25vam95IiwiZmFtaWx5IjoiSGVtIn19XSwiaWRzIjp7Imlzc24iOlsiMTY2NC0zMDJYIl0sImlzYm4iOltdLCJkb2kiOiIxMC4zMzg5L2ZtaWNiLjIwMjQuMTQ2NDY4NiIsInBtaWQiOiIzOTU2ODk5NyIsIm9wZW5hbGV4IjoiVzQ0MDQxMjA5NTIiLCJleGEiOiJodHRwczovL3d3dy5mcm9udGllcnNpbi5vcmcvam91cm5hbHMvbWljcm9iaW9sb2d5L2FydGljbGVzLzEwLjMzODkvZm1pY2IuMjAyNC4xNDY0Njg2L2Z1bGwifSwidGl0bGUiOiJTZXdhZ2UtYW5kIGZlcnRpbGl6ZXItZGVyaXZlZCBudXRyaWVudHMgYWx0ZXIgdGhlIGludGVuc2l0eSwgZGl2ZXJzaXR5LCBhbmQgdG94aWNpdHkgb2YgaGFybWZ1bCBjeWFub2JhY3RlcmlhbCBibG9vbXMgaW4gZXV0cm9waGljIGxha2VzIiwiYWJzdHJhY3QiOiJDeWFub2JhY3RlcmlhbCBoYXJtZnVsIGFsZ2FsIGJsb29tcyAoQ0hBQnMpIGFyZSBwcm9tb3RlZCBieSBleGNlc3NpdmUgbnV0cmllbnQgbG9hZGluZyBhbmQsIHdoaWxlIGZlcnRpbGl6ZXJzIGFuZCBzZXdhZ2UgYXJlIHRoZSBtb3N0IHByZXZhbGVudCBleHRlcm5hbCBudXRyaWVudCBzb3VyY2VzIGluIG1vc3Qgd2F0ZXJzaGVkcywgdGhlIGRpZmZlcmVudGlhbCBlZmZlY3RzIG9mIHRoZXNlIG51dHJpZW50IHNvdXJjZXMgb24gQ0hBQnMgYXJlIHVua25vd24uIEhlcmUsIHdlIHRyYWNrZWQgQ0hBQnMgYW5kIHBlcmZvcm1lZCBleHBlcmltZW50cyBpbiBmaXZlIGRpc3RpbmN0IGxha2VzIGFjcm9zcyB0aGUgTm9ydGhlcm4gVVMgaW5jbHVkaW5nIExha2UgRXJpZS4gRmVydGlsaXplcnMgd2l0aCBhbW1vbml1bSBhbmQgb3J0aG9waG9zcGhhdGUsIG1lbWJyYW5lICgwLjIgzrxtKS1maWx0ZXJlZCBzZXdhZ2UgKGRvbWluYXRlZCBieSByZWR1Y2VkIGZvcm1zIG9mIG5pdHJvZ2VuKSBzYW5kLWFuZCBtZW1icmFuZS1maWx0ZXJlZCBzZXdhZ2UgKGRvbWluYXRlZCBieSBuaXRyYXRlKSwgYW5kIGFuIGlub3JnYW5pYyBudXRyaWVudCBzb2x1dGlvbiBvZiBhbW1vbml1bSBhbmQgb3J0aG9waG9zcGhhdGUgd2VyZSB1c2VkIGFzIGV4cGVyaW1lbnRhbCBudXRyaWVudCBzb3VyY2VzIGZvciBDSEFCcyBhdCBOLWVxdWl2YWxlbnQsIGVudmlyb25tZW50YWxseSByZWFsaXN0aWMgY29uY2VudHJhdGlvbnMuIFBoeXRvcGxhbmt0b24gY29tbXVuaXRpZXMgd2VyZSBldmFsdWF0ZWQgZmx1b3JvbWV0cmljYWxseSwgbWljcm9zY29waWNhbGx5LCBhbmQgdmlhIGhpZ2ggdGhyb3VnaHB1dCBzZXF1ZW5jaW5nIG9mIHRoZSAxNlMgclJOQSBnZW5lLCBhbmQgbGV2ZWxzIG9mIG1pY3JvY3lzdGluIGFuZCB0aGUgzrQgMTUgTiBjb250ZW50IG9mIHBhcnRpY3VsYXRlIG9yZ2FuaWMgbml0cm9nZW4gKM60UE8gMTUgTikgd2VyZSBxdWFudGlmaWVkLiBGZXJ0aWxpemVyIGFuZCBib3RoIHNvdXJjZXMgb2Ygd2FzdGV3YXRlciBpbmNyZWFzZWQgdGhlIGFidW5kYW5jZSBvZiBjeWFub2JhY3RlcmlhIGluIGFsbCBleHBlcmltZW50cyBhY3Jvc3MgYWxsIGZpdmUgbGFrZXMgKCBwICZhbXA7bHQ7IDAuMDUgZm9yIGFsbCkgd2hlcmVhcyBlZmZlY3RzIG9uIGV1a2FyeW90aWMgcGh5dG9wbGFua3RvbiB3ZXJlIGxpbWl0ZWQuIFNhbmQtZmlsdGVyZWQgc2V3YWdlIGNvbnRhaW5lZCBsZXNzIFAsIG9yZ2FuaWMgbWF0dGVyLCBhbmQgYW1tb25pdW0gYnV0IG1vcmUgbml0cmF0ZSBhbmQgaGFkIGEgMjUlIGxlc3MgcG90ZW50IHN0aW11bGF0b3J5IGVmZmVjdCBvbiBjeWFub2JhY3RlcmlhIHRoYW4gbWVtYnJhbmUtZmlsdGVyZWQgc2V3YWdlLCBzdWdnZXN0aW5nIG5pdHJpZmljYXRpb24gbWF5IHBsYXkgYSByb2xlIGluIHJlZHVjaW5nIENIQUJzLiBGZXJ0aWxpemVyIGluY3JlYXNlZCBtaWNyb2N5c3RpbiBsZXZlbHMgYW5kIGRlY3JlYXNlZCB0aGUgzrRQTyAxNSBOIHdoZXJlYXMgd2FzdGV3YXRlciBpbmNyZWFzZWQgzrRQTyAxNSBOICggcCAmYW1wO2x0OyAwLjA1IGZvciBhbGwpLiBNaWNyb2N5c3RpcyB3YXMgdGhlIGdlbnVzIG1vc3QgY29uc2lzdGVudGx5IHByb21vdGVkIGJ5IG51dHJpZW50IHNvdXJjZXMgKCBwICZhbXA7bHQ7IDAuMDUgaW4gYWxsIGV4cGVyaW1lbnRzKSwgZm9sbG93ZWQgYnkgQ3lhbm9iaXVtICggcCAmYW1wO2x0OyAwLjA1IGluIDUwJSBvZiBleHBlcmltZW50cyksIHdpdGggaW5jcmVhc2VzIGluIE1pY3JvY3lzdGlzIGJpb21hc3MgY29uc2lzdGVudGx5IGVsaWNpdGVkIGJ5IG1lbWJyYW5lLWZpbHRlcmVkIHdhc3Rld2F0ZXIuIENvbGxlY3RpdmVseSwgcmVzdWx0cyBkZW1vbnN0cmF0ZSB0aGF0IGRpZmZlcmluZyB0eXBlcyBvZiBzZXdhZ2UgZGlzY2hhcmdlIGFuZCBmZXJ0aWxpemVycyBjYW4gcHJvbW90ZSBDSEFCIGludGVuc2l0eSBhbmQgdG94aWNpdHksIHdoaWxlIGNvbmN1cnJlbnRseSBhbHRlcmluZyBDSEFCIGRpdmVyc2l0eSBhbmQgzrRQTyAxNSBOLiBXaGlsZSBtZW1icmFuZS1maWx0ZXJlZCBzZXdhZ2UgY29uc2lzdGVudGx5IGZhdm9yZWQgTWljcm9jeXN0aXMgLCB0aGUgZGlzY2hhcmdlIG9mIHNld2FnZSB0aHJvdWdoIHNhbmRzIG11dGVkIGJsb29tIGludGVuc2l0eSBzdWdnZXN0aW5nIHNhbmQtYmVkcyBtYXkgcmVwcmVzZW50IGEgdG9vbCB0byByZW1vdmUga2V5IG51dHJpZW50cyBhbmQgcGFydGlhbGx5IG1pdGlnYXRlIENIQUJzLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzg5L2ZtaWNiLjIwMjQuMTQ2NDY4NiIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cuZnJvbnRpZXJzaW4ub3JnL2pvdXJuYWxzL21pY3JvYmlvbG9neS9hcnRpY2xlcy8xMC4zMzg5L2ZtaWNiLjIwMjQuMTQ2NDY4Ni9wZGYiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMS0wNiIsImFjY2Vzc2VkRGF0ZSI6IjIwMjYtMDIiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZyb250aWVycyBpbiBNaWNyb2Jpb2xvZ3kiLCJ2b2x1bWUiOjE1LCJpbXBhY3RGYWN0b3IiOiIzLjk5In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTQ2NDY4NiwibGFzdFBhZ2UiOjE0NjQ2ODZ9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRnJvbnRpZXJzIE1lZGlhIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo3MywiY2l0ZWRCeUNvdW50Ijo5fX0sInF1b3RlIjoiZGVyaXZlZCBudXRyaWVudHMgYWx0ZXIgdGhlIGludGVuc2l0eSwgZGl2ZXJzaXR5LCBhbmQgdG94aWNpdHkgb2YgaGFybWZ1bCBjeWFub2JhY3RlcmlhbCBibG9vbXMgLi4uIEN5YW5vYmFjdGVyaWFsIGhhcm1mdWwgYWxnYWwgYmxvb21zIChDSEFCcykgYXJlIHByb21vdGVkIGJ5IGV4Y2Vzc2l2ZSBudXRyaWVudCBsb2FkaW5nIGFuZCwgd2hpbGUgZmVydGlsaXplcnMgYW5kIHNld2FnZSBhcmUgdGhlIG1vc3QgcHJldmFsZW50IGV4dGVybmFsIG51dHJpZW50IHNvdXJjZXMgaW4gbW9zdCB3YXRlcnNoZWRzLCB0aGUgZGlmZmVyZW50aWFsIGVmZmVjdHMgb2YgdGhlc2UgbnV0cmllbnQgLi4uIG9yZ2FuaWMgbml0cm9nZW4gKM60UE8xNU4pIHdlcmUgcXVhbnRpZmllZC4gRmVydGlsaXplciBhbmQgYm90aCBzb3VyY2VzIG9mIHdhc3Rld2F0ZXIgaW5jcmVhc2VkIHRoZSBhYnVuZGFuY2Ugb2YgLi4uIHBoeXRvcGxhbmt0b24gd2VyZSBsaW1pdGVkLiBTYW5kLWZpbHRlcmVkIHNld2FnZSBjb250YWluZWQgbGVzcyBQLCBvcmdhbmljIG1hdHRlciwgYW5kIGFtbW9uaXVtIGJ1dCBtb3JlIG5pdHJhdGUgYW5kIC4uLiBkaWZmZXJpbmcgdHlwZXMgb2Ygc2V3YWdlIGRpc2NoYXJnZSBhbmQgZmVydGlsaXplcnMgY2FuIHByb21vdGUgQ0hBQiBpbnRlbnNpdHkgYW5kIHRveGljaXR5LCB3aGlsZSBjb25jdXJyZW50bHkgLi4uIHNld2FnZSB0aHJvdWdoIHNhbmRzIG11dGVkIGJsb29tIGludGVuc2l0eSBzdWdnZXN0aW5nIHNhbmQtYmVkcyBtYXkgcmVwcmVzZW50IGEgdG9vbCB0byByZW1vdmUga2V5IG51dHJpZW50cyBhbmQgLi4uICggV2F0c29uIGV0IGFsLiwgMjAxNiA7IEJ1bGxlcmphaG4gZXQgYWwuLCAyMDE2IDsgSHVpc21hbiBldCBhbC4sIDIwMTggKSBhbmQgZXhjZXNzaXZlIG51dHJpZW50IGxvYWRpbmcgaXMgdGhlIHByb2Nlc3MgbW9zdCBjb21tb25seSBjaXRlZCBhcyB0aGUgZW52aXJvbm1lbnRhbCBmYWN0b3IgcHJvbW90aW5nIENIQUJzICgifV0%3D.

Mitigating this demands bridging the sewage treatment deficit through decentralized plants, sewerage networks, and industrial zero-liquid discharge mandates. Nature-based solutions like constructed wetlands could assimilate 50-70% of nutrients pre-inflow, while precision monitoring via citizen science tracks compliance (Kumar & Mahajan, 2020; Shahbaz et al., 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlBhd2FuIiwiZmFtaWx5IjoiS3VtYXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW1icmlzaCBLdW1hciIsImZhbWlseSI6Ik1haGFqYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyNTIzLTM5NjMiLCIyNTIzLTM5NzEiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMDcvczQyNDUyLTAyMC0zMDgyLTgiLCJtYWciOiIzMDM1OTYwMjI4Iiwib3BlbmFsZXgiOiJXMzAzNTk2MDIyOCJ9LCJ0aXRsZSI6IlRyb3BoaWMgc3RhdHVzIGFuZCBpdHMgcmVndWxhdGluZyBmYWN0b3IgZGV0ZXJtaW5hdGlvbiBhdCB0aGUgUmV3YWxzYXIgTGFrZSwgbm9ydGh3ZXN0IEhpbWFsYXlhIChIUCksIEluZGlhLCBiYXNlZCBvbiBzZWxlY3RlZCBwYXJhbWV0ZXJzIGFuZCBtdWx0aXZhcmlhdGUgc3RhdGlzdGljYWwgYW5hbHlzaXMiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAwNy9zNDI0NTItMDIwLTMwODItOCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9saW5rLnNwcmluZ2VyLmNvbS9jb250ZW50L3BkZi8xMC4xMDA3L3M0MjQ1Mi0wMjAtMzA4Mi04LnBkZiIsImlzc3VlZERhdGUiOiIyMDIwLTA2LTIyIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiU04gQXBwbGllZCBTY2llbmNlcyIsImlzc3VlIjo3LCJ2b2x1bWUiOjIsImltcGFjdEZhY3RvciI6IjcuMzUifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiU3ByaW5nZXIgTmF0dXJlIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo0MCwiY2l0ZWRCeUNvdW50IjoxOX19LCJxdW90ZSI6IkNoYW5naW5nIHF1YWxpdHkgaGFzIGNyZWF0ZWQgdGhlIHdhdGVyIHNjYXJjaXR5IGFuZCAgYSBodWdlIG1hc3Mgb2YgcG9wdWxhdGlvbiB0aHVzIGJlZW4gZGVwcml2ZWQgZnJvbSB1c2luZyAgdGhlIGZyZXNod2F0ZXIgZm9yIHZhcmlvdXMgcHVycG9zZXMgWzMsIDRdLiBIb3dldmVyLCB0aGUgIG9ibm94aW91cyBtYXRlcmlhbHMgdGhhdCBzdGF5IGluIHdhdGVyIGJvZGllcyBmb3IgYSBsb25nLXRpbWUgcGVyaW9kIGJyaW5nIGNoYW5nZXMgaW4gdGhlIHBoeXNpb2NoZW1pY2FsIGNoYXItYWN0ZXJpc3RpY3MgYXMgd2VsbCBhcyB0aGUgbmF0dXJhbCBzY2VuYXJpbyBvZiB0aGUgbGFrZXMgWzVdLiAgVGhlIEhpbWFsYXlhbiBsYWtlcyBhcmUgdW5kZXIgc3RyZXNzIGR1ZSB0byBtaXNtYW5hZ2VkIGh1bWFuIGFjdGl2aXRpZXMsIHdoaWNoIGluY3JlYXNlIG51dHJpZW50cyBhbmQgcG9sbHV0aW9uIGluIHRlcm1zIG9mIGhlYXZ5IG1ldGFscyB0b3hpY2l0eSBhbmQgYmFjdGVyaWFsIGNvbnRhbWluYXRpb24gIFs24oCTOF0uIFRoZSBtYW5hZ2VtZW50IGFuZCBwcmVzZXJ2YXRpb24gb2YgdGhlIGxha2VzIGF0ICBwcmVzZW50IGhhdmUgYmVjb21lIGEgZ3JlYXQgY2hhbGxlbmdlIGZvciBlbnZpcm9ubWVudGFsLWlzdHMgYW5kIG1hbmFnZXJzIFs1LCA5XS4gVGhlIGV1dHJvcGhpY2F0aW9uIGlzIGFsc28gYSBtYWluIHByb2JsZW0gb2YgdGhlICBmcmVzaHdhdGVyIGxha2VzLCBhbmQgaXQgaXMgc3BlY2lmaWVkIGZvciB0aGUgbGFrZXMgaGF2aW5nICBudXRyaWVudCBlbnJpY2htZW50LiBJdCBjYXVzZXMgZXhjZXNzaXZlIGdyb3d0aCBvZiBhbGdhZSAgYW5kIGlzIGNvbnNpZGVyZWQgdG8gYmUgcmVzcG9uc2libGUgZm9yIGRlZ3JhZGluZyB0aGUgIHF1YWxpdGF0aXZlIG5hdHVyZSBvZiB0aGUgbGFrZXMgWzUsIDEwXS4gVGhlIGV1dHJvcGhpY2F0aW9uICBpcyBmb2xsb3dlZCBieSB0aGUgYWxnYWwgYmxvb20sIHdoaWNoIGdlbmVyYXRlcyB0b3hpbiwgIHdvcnNlbnMgd2F0ZXIgcXVhbGl0eSwgZGVwbGV0ZXMgZGlzc29sdmVkIG94eWdlbiAoRE8pICogUGF3YW4gS3VtYXIsIHBhd2FuLmV2c0BnbWFpbC5jb20gfCAxRGVwYXJ0bWVudCBvZiBFbnZpcm9ubWVudGFsIFNjaWVuY2VzLCBDZW50cmFsIFVuaXZlcnNpdHkgb2YgSGltYWNoYWwgUHJhZGVzaCwgRGhhcmFtc2hhbGEgMTc2MjE1LCBJbmRpYS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2OC41Nzg0NjA2OTMzNTkzOCwibGVmdCI6OC41NzE0MTg3NjIyMDcwMzEsIndpZHRoIjo0MC4zMjc1MTQ2NDg0Mzc1LCJoZWlnaHQiOjEuNTE5ODU1MjYwODQ4OTk5fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NzAuMTU4OTk2NTgyMDMxMjUsImxlZnQiOjguNTY5NzM4Mzg4MDYxNTIzLCJ3aWR0aCI6NDAuMzU0NzI0ODg0MDMzMiwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjcxLjczOTU0NzcyOTQ5MjE5LCJsZWZ0Ijo4LjU2OTczODM4ODA2MTUyMywid2lkdGgiOjQwLjM0OTY5MzI5ODMzOTg0NCwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjczLjMyMDA5MTI0NzU1ODYsImxlZnQiOjguNTY4MDU5OTIxMjY0NjQ4LCJ3aWR0aCI6NDAuMDIwMDg4MTk1ODAwNzgsImhlaWdodCI6MS41MTk4NTUyNjA4NDg5OTl9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo3NC45MDA2NDIzOTUwMTk1MywibGVmdCI6OC41NjgwNTk5MjEyNjQ2NDgsIndpZHRoIjozOS45OTk5Mjc1MjA3NTE5NSwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjc2LjQ4MTE4NTkxMzA4NTk0LCJsZWZ0Ijo4LjU2ODA1NzA2MDI0MTcsIndpZHRoIjo0MC4zNTcwNzQ3Mzc1NDg4MywiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjc4LjA2MTcyOTQzMTE1MjM0LCJsZWZ0Ijo4LjU2OTczNjQ4MDcxMjg5LCJ3aWR0aCI6NDAuMzUzNzEzOTg5MjU3ODEsImhlaWdodCI6MS41MTk4NTUyNjA4NDg5OTl9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2MC42NzU3Mjc4NDQyMzgyOCwibGVmdCI6NTEuNDI3MTY1OTg1MTA3NDIsIndpZHRoIjo0MC4zNzg1NjY3NDE5NDMzNiwiaGVpZ2h0IjoxOC45MDU4NTUxNzg4MzMwMDh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo4MS45MjczMzc2NDY0ODQzOCwibGVmdCI6OC40MDE0OTc4NDA4ODEzNDgsIndpZHRoIjo3My45NTQ2OTY2NTUyNzM0NCwiaGVpZ2h0IjoyLjY3ODc2MjY3NDMzMTY2NX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEifX0seyJvcmlnaW4iOjQsIndvcmsiOnsidHlwZSI6OCwic3VidHlwZSI6eyJvbmVvZktpbmQiOiJhcnRpY2xlVHlwZSIsImFydGljbGVUeXBlIjoxfSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTWQiLCJmYW1pbHkiOiJTaGFoYmF6In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFoYmFyIiwiZmFtaWx5IjoiQWxhbSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNb2hhbW1hZCBNYXNyb29yIiwiZmFtaWx5IjoiWmFmYXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTW9oYW1tZWQgQWFzaWYiLCJmYW1pbHkiOiJTdWxhaW1hbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBbnVwbWEiLCJmYW1pbHkiOiJLdW1hcmkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiRy4gSy4iLCJmYW1pbHkiOiJTaGFybWEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2hhaGxhIiwiZmFtaWx5IjoiWWFzbWluIn19XSwiaWRzIjp7Imlzc24iOlsiMTIxNS0xNjEwIiwiMjA2MS05NTg4Il0sImlzYm4iOltdLCJkb2kiOiIxMC4yNDc4L29yaHUtMjAyMy0wMDE2Iiwib3BlbmFsZXgiOiJXNDM4OTUxMDAzNyJ9LCJ0aXRsZSI6IkNvbXBhcmlzb24gb2YgYXZpYW4gZGl2ZXJzaXR5IGJldHdlZW4gbWFuYWdlZCBhbmQgdW5tYW5hZ2VkIHdldGxhbmRzIGluIFBhdG5hLCBCaWhhciwgSW5kaWEiLCJhYnN0cmFjdCI6IkFic3RyYWN0IFdlIGNvbXBhcmVkIHRoZSBiaXJkIGRpdmVyc2l0eSBvZiBhIG1hbmFnZWQgKFJhamRoYW5pIEphbGFzaGF5KSBhbmQgYW4gdW5tYW5hZ2VkIHdldGxhbmQgKE11c3RhZmFwdXIpIGluIFBhdG5hLCBCaWhhciwgSW5kaWEuIFdlIHJlY29yZGVkIGJpcmQgc3BlY2llcyBieSB1c2luZyBwb2ludCBjb3VudHMgYW5kIGZpeGVkLXJvdXRlIG1vbml0b3JpbmcuIEF0IFJhamRoYW5pIEphbGFzaGF5LCBhIHRvdGFsIG9mIDczIHNwZWNpZXMgd2VyZSByZWNvcmRlZCwgb2Ygd2hpY2ggMTUgc3BlY2llcyB3ZXJlIHdpbnRlciB2aXNpdG9ycy4gVHdvIHNwZWNpZXMgd2VyZSBpbiB0aGUgTmVhci10aHJlYXRlbmVkIGNhdGVnb3J5OiBGZXJydWdpbm91cyBEdWNrIChBeXRoeWEgbnlyb2NhKSBhbmQgQWxleGFuZHJpbmUgUGFyYWtlZXQgKFBzaXR0YWN1bGEgZXVwYXRyaWEpICwgd2hpbGUgdGhlIG90aGVycyB3ZXJlIG9mIGxlYXN0IGNvbmNlcm4uIEF0IE11c3RhZmFwdXIgd2V0bGFuZCwgNjcgc3BlY2llcyB3ZXJlIHJlY29yZGVkIHdpdGggMTEgc3BlY2llcyBhcyB3aW50ZXIgdmlzaXRvcnMuIFNpbXBzb27igJlzIHNwZWNpZXMgZGl2ZXJzaXR5IGluZGV4IHdhcyAwLjc4IGF0IFJhamRoYW5pIEphbGFzaGF5LCBhbmQgMC44MSBhdCBNdXN0YWZhcHVyIHdldGxhbmQuIFRoZSBudW1iZXIgb2Ygc3BlY2llcyBvZiB3YWRlcnMgd2FzIDE3IGF0IE11c3RhZmFwdXIgd2V0bGFuZCBhbmQgMTEgYXQgUmFqZGhhbmkgSmFsYXNoYXkuIFRoZSB0b3RhbCBudW1iZXIgb2YgTGVzc2VyIFdoaXN0bGluZy1kdWNrIChEZW5kcm9jeWduYSBqYXZhbmljYSkgLCBhIHJlc2lkZW50IGJpcmQgd2FzIGFsbW9zdCBkb3VibGUgKDUyNikgYXQgTXVzdGFmYXB1ciB3ZXRsYW5kIHRoYW4gdGhhdCBhdCBSYWpkaGFuaSBKYWxhc2hheSAoMjM0KS4gVGhlIG5pdHJhdGUgbGV2ZWwgd2FzIHR3aWNlIGhpZ2hlciBhbmQgdGhlIHBob3NwaGF0ZSBsZXZlbCB3YXMgZm91ciB0aW1lcyBoaWdoZXIgYXQgTXVzdGFmYXB1ciB3ZXRsYW5kIHRoYW4gdGhvc2UgYXQgUmFqZGhhbmkgSmFsYXNoYXkuIFRoZSBkaWZmZXJlbmNlIGluIHNwZWNpZXMgbnVtYmVyIGFuZCBjb21wb3NpdGlvbiBiZXR3ZWVuIHRoZSB0d28gd2V0bGFuZHMgbWF5IGJlIGF0dHJpYnV0ZWQgdG8gdGhlIGV1dHJvcGhpY2F0aW9uIHJlc3VsdGluZyBmcm9tIGhpZ2ggbml0cmF0ZSBhbmQgcGhvc3BoYXRlIGxldmVscyBhdCBNdXN0YWZhcHVyIHdldGxhbmQuIFVuZm9ydHVuYXRlbHksIHRoZSBNdXN0YWZhcHVyIHdldGxhbmQgaXMgdW5kZXIgdGhyZWF0IGR1ZSB0byBodW1hbiBpbmZsdWVuY2VzLiBUaGUgcHJvdGVjdGlvbiBhbmQgc3VzdGFpbmFibGUgbWFuYWdlbWVudCBvZiBuYXR1cmFsIHdldGxhbmRzIGlzIHJlcXVpcmVkIGZvciBzYXZpbmcgdGhlIGJpb2RpdmVyc2l0eSBvZiB0aGUgYXJlYS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjQ3OC9vcmh1LTIwMjMtMDAxNiIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9zY2llbmRvLmNvbS9wZGYvMTAuMjQ3OC9vcmh1LTIwMjMtMDAxNiIsImlzc3VlZERhdGUiOiIyMDIzLTEyLTAxIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0wOSIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiT3JuaXMgSHVuZ2FyaWNhIiwiaXNzdWUiOjIsInZvbHVtZSI6MzEsImltcGFjdEZhY3RvciI6IjAuOTIifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjoxMywibGFzdFBhZ2UiOjI4fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6Ikh1bmdhcmlhbiBPcm5pdGhvbG9naWNhbCBhbmQgTmF0dXJlIENvbnNlcnZhdGlvbiBTb2NpZXR5In0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjoyNywiY2l0ZWRCeUNvdW50IjoxfX0sInF1b3RlIjoiSG93ZXZlciwgaGlnaCBsZXZlbHMgb2YgZXV0cm9waGljYXRpb24gbWF5IGFsc28gdHJpZ2dlciB0aGUgcHJvbGlmZXJhdGlvbiBvZiBncmVlbiBhbGdhZSBhbmQgIGRlY2xpbmUgb2YgbWFjcm9waHl0ZXMgbGVhZGluZyB0byBhIGhpZ2gtdHVyYmlkaXR5IHN0YXRlIG9mIHdhdGVyLCBpbiB3aGljaCBmZXcgc3BlY2llcyBjYW4gIHN1cnZpdmUgKFNjaGVmZmVyIGV0IGFsLiAxOTkzKS4gVG8gYWRkcmVzcyB0aGUgZXV0cm9waGljYXRpb24gYW5kIGV4Y2VzcyBudXRyaWVudCBpc3N1ZXMgaW4gIGFxdWF0aWMgZWNvc3lzdGVtcywgcGh5dG9yZW1lZGlhdGlvbiBvZmZlcnMgYSBwcm9taXNpbmcgc29sdXRpb24gKExpdSBldCBhbC4gMjAyMSkuIEFxdWF0aWMgIHBsYW50cyBsaWtlIGR1Y2t3ZWVkcywgd2F0ZXIgbGV0dHVjZSAoUGlzdGlhIHN0cmF0aW90ZXMpLCBhbmQgd2F0ZXIgbGlsaWVzIGhhdmUgZGVtb25zdHJhdGVkICB0aGVpciBhYmlsaXR5IHRvIGNvbWJhdCBldXRyb3BoaWNhdGlvbiBieSBhYnNvcmJpbmcgYW5kIGFzc2ltaWxhdGluZyBleGNlc3MgcGhvc3Bob3J1cyBhbmQgIG5pdHJvZ2VuIChOYWhhciAmIEhvcXVlIDIwMjEpLiBIb3dldmVyLCB0aGVzZSBwbGFudHMgY2FuIHNvbWV0aW1lcyBncm93IHVuY29udHJvbGxhYmx5LCAgcG90ZW50aWFsbHkgY292ZXJpbmcgdGhlIHdhdGVyIHN1cmZhY2VzLiBUbyBhdm9pZCBzdWNoIHNjZW5hcmlvcywgcmVndWxhciBtb25pdG9yaW5nIGlzIGVzc2VudGlhbCAoTGl1IGV0IGFsLiAyMDIxKS4gUHJvbW90aW5nIHB1YmxpYyBhd2FyZW5lc3MgaXMgdGhlIGNvcm5lcnN0b25lIGZvciBtb2JpbGl6aW5nIHN1cHBvcnQgYW5kIGVuc3VyaW5nIGVmZmVjdGl2ZSBldXRyb3BoaWNhdGlvbiBtYW5hZ2VtZW50LiBDb250cm9sbGVkIGVyYWRpY2F0aW9uIG9mIGV4Y2VzcyAgZmxvcmEgd2hlbiBuZWNlc3NhcnkgaXMgdml0YWwgdG8gcHJldmVudCBvdmVyZ3Jvd3RoLiBUaGlzIGludGVncmF0ZWQgYXBwcm9hY2ggY2FuIGVmZmVjdGl2ZWx5ICByZWR1Y2UgZXV0cm9waGljYXRpb24gaW4gdGhlIHdhdGVyIGJvZGllcyAoV2FuZyBldCBhbC4gMjAyMikuIEVhY2ggc3BlY2llcyBvZiB3ZXRsYW5kLWRlcGVuZGVudCBiaXJkIGhhcyBhIHVuaXF1ZSBhbmQgY29tcGxleCBzZXQgb2YgbmVlZHMgZm9yIHdldGxhbmQgaGFiaXRhdHMgdGhhdCBtYWtlcyBpdCBkaWZmaWN1bHQgdG8gZ2VuZXJhbGl6ZSBhYm91dCBob3cgbG9zcyBvciBkZWdyYWRhdGlvbiBvZiB3ZXRsYW5kcyBhZmZlY3RzIGJpcmQgcG9wdWxhdGlvbnMuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjc5LjI5NTYzOTAzODA4NTk0LCJsZWZ0IjoxMC4zMDMwMDYxNzIxODAxNzYsIndpZHRoIjo3OS45NTI3MjgyNzE0ODQzOCwiaGVpZ2h0IjoxLjcwOTE5Nzg3ODgzNzU4NTR9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6ODEuMjEyNTM5NjcyODUxNTYsImxlZnQiOjEwLjMwMzAwNjE3MjE4MDE3Niwid2lkdGgiOjc5Ljk1OTI4MTkyMTM4NjcyLCJoZWlnaHQiOjEuNzA5MTk3ODc4ODM3NTg1NH19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo4My4xMjk0NDc5MzcwMTE3MiwibGVmdCI6MTAuMzAzMDA2MTcyMTgwMTc2LCJ3aWR0aCI6NzkuOTQyNzI2MTM1MjUzOSwiaGVpZ2h0IjoxLjcwOTE5Nzg3ODgzNzU4NTR9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6ODUuMDQ2MzQ4NTcxNzc3MzQsImxlZnQiOjEwLjMwMzAwODA3OTUyODgwOSwid2lkdGgiOjc5LjkzOTQ1MzEyNSwiaGVpZ2h0IjoxLjcwOTE5Nzg3ODgzNzU4NTR9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6ODYuOTYzMjU2ODM1OTM3NSwibGVmdCI6MTAuMzAzMDEwOTQwNTUxNzU4LCJ3aWR0aCI6NzkuOTQxODQ4NzU0ODgyODEsImhlaWdodCI6MS43MDkxOTc4Nzg4Mzc1ODU0fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjg4Ljg4MDE2NTEwMDA5NzY2LCJsZWZ0IjoxMC4zMDMwMTM4MDE1NzQ3MDcsIndpZHRoIjo3OS45Mzc1LCJoZWlnaHQiOjEuNzA5MTk3ODc4ODM3NTg1NH19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo5MC43OTcwNjU3MzQ4NjMyOCwibGVmdCI6MTAuMzAzMDEzODAxNTc0NzA3LCJ3aWR0aCI6NzkuOTM5NjUxNDg5MjU3ODEsImhlaWdodCI6MS43MDkxOTc4Nzg4Mzc1ODU0fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjkyLjcxMzk3Mzk5OTAyMzQ0LCJsZWZ0IjoxMC4zMDMwMTM4MDE1NzQ3MDcsIndpZHRoIjo3OS45NTAzMjUwMTIyMDcwMywiaGVpZ2h0IjoxLjcwOTE5Nzg3ODgzNzU4NTR9fSx7InBhZ2VOdW1iZXIiOjEzLCJiYm94Ijp7InRvcCI6MTIuMjA3OTkwNjQ2MzYyMzA1LCJsZWZ0IjoxMC4zMDMwMjQyOTE5OTIxODgsIndpZHRoIjo3OS45NTI1MTQ2NDg0Mzc1LCJoZWlnaHQiOjcuNDU5OTIyNzkwNTI3MzQ0fX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjE5Ljg3NTYyNTYxMDM1MTU2MiwibGVmdCI6MTIuMTIxMzI0NTM5MTg0NTcsIndpZHRoIjo3OC4xMTkyMjQ1NDgzMzk4NCwiaGVpZ2h0IjoxLjcwOTE5Nzg3ODgzNzU4NTR9fSx7InBhZ2VOdW1iZXIiOjEzLCJiYm94Ijp7InRvcCI6MjEuNzkyNTMzODc0NTExNzIsImxlZnQiOjEwLjMwMzAyOTA2MDM2Mzc3LCJ3aWR0aCI6NzkuOTQ5MjQ5MjY3NTc4MTIsImhlaWdodCI6MS43MDkxOTc4Nzg4Mzc1ODU0fX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjIzLjcwOTQzNjQxNjYyNTk3NywibGVmdCI6MTAuMzAzMDI5MDYwMzYzNzcsIndpZHRoIjo3OS45NTIyNzA1MDc4MTI1LCJoZWlnaHQiOjEuNzA5MTk3ODc4ODM3NTg1NH19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjI1In19XQ%3D%3D. Integrating riparian buffers around lakes would curb direct discharges, restoring stoichiometric balance and curbing HABs before 90% of urban lakes become unusable (Jilbert et al., 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRvbSIsImZhbWlseSI6IkppbGJlcnQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUmFvdWzigJBNYXJpZSIsImZhbWlseSI6IkNvdXR1cmUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQnJpYW4iLCJmYW1pbHkiOiJIdXNlciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLYWxldmkiLCJmYW1pbHkiOiJTYWxvbmVuIn19XSwiaWRzIjp7Imlzc24iOlsiMDAxOC04MTU4IiwiMTU3My01MTE3Il0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDA3L3MxMDc1MC0wMjAtMDQ0NTcteCIsIm1hZyI6IjMxMDA0NTkwNTAiLCJvcGVuYWxleCI6IlczMTAwNDU5MDUwIn0sInRpdGxlIjoiUHJlZmFjZTogUmVzdG9yYXRpb24gb2YgZXV0cm9waGljIGxha2VzOiBjdXJyZW50IHByYWN0aWNlcyBhbmQgZnV0dXJlIGNoYWxsZW5nZXMiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAwNy9zMTA3NTAtMDIwLTA0NDU3LXgiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vbGluay5zcHJpbmdlci5jb20vY29udGVudC9wZGYvMTAuMTAwNy9zMTA3NTAtMDIwLTA0NDU3LXgucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjAtMTEtMTYiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA3IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJIeWRyb2Jpb2xvZ2lhIiwiaXNzdWUiOjIxLCJ2b2x1bWUiOjg0NywiaW1wYWN0RmFjdG9yIjoiMy4wMCJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjQzNDMsImxhc3RQYWdlIjo0MzU3fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlNwcmluZ2VyIFNjaWVuY2UrQnVzaW5lc3MgTWVkaWEifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjk1LCJjaXRlZEJ5Q291bnQiOjU4fX0sInF1b3RlIjoiU2ltaWxhciBkZWdyYWRhdGlvbiBvZiBmcmVzaHdhdGVyIGVjb3N5c3RlbXMgaXMgb2JzZXJ2ZWQgd29ybGR3aWRlLCB3aXRoIHRoZSB0aW1lbGluZSBvZiBodW1hbiBwcmVzc3VyZSBkaWN0YXRlZCBieSB0cmVuZHMgaW4gdXJiYW5pemF0aW9uIGFuZCBpbnRlbnNpdmUgYWdyaWN1bHR1cmUgaW4gZGlmZmVyZW50IGdlb2dyYXBoaWMgcmVnaW9ucyAoZS5nLiwgVmVubmVtbyBldCBhbC4sIDIwMDkpLiBUaGUgcHJpbmNpcGFsIGRyaXZlciBvZiBlY29zeXN0ZW0gc3RyZXNzIGluIG1vc3QgYXF1YXRpYyBzeXN0ZW1zIGlzIGV1dHJvcGhpY2F0aW9uLCB0aGUgZXhjZXNzIGdyb3d0aCBvZiBhbGdhZSBkcml2ZW4gYnkgYW50aHJvcG9nZW5pYyBpbnB1dHMgb2YgdGhlIG1hY3JvbnV0cmllbnRzIG5pdHJvZ2VuIChOKSBhbmQgcGhvc3Bob3J1cyAoUCkgKFNtaXRoLCAyMDAzOyBCaXJrIGV0IGFsLiwgMjAyMCkuIE51dHJpZW50cyBhcmUgZW1pdHRlZCBmcm9tIGJvdGggcG9pbnQgYW5kIGRpZmZ1c2Ugc291cmNlcyBpbiB0aGUgaHVtYW4gZW52aXJvbm1lbnQgYW5kIGVudGVyIGFxdWF0aWMgc3lzdGVtcyB2aWEgc3VyZmFjZSBydW5vZmYsIGFzIHdlbGwgYXMgZ3JvdW5kLXdhdGVyIGFuZCBhdG1vc3BoZXJpYyBkZXBvc2l0aW9uIChQYWVybCwgMTk5NykuIEV1dHJvcGhpY2F0aW9uIG9mdGVuIGxlYWRzIHRvIHRoZSBkb21pbmFuY2Ugb2YgcHJpbWFyeSBwcm9kdWN0aW9uIGJ5IGN5YW5vYmFjdGVyaWEsIGluY2x1ZGluZyB0b3hpYyBnZW5lcmEgc3VjaCBhcyBEb2xpY2hvc3Blcm11bSwgQXBoYW5pem9tZW5vbiwgYW5kIE1pY3JvY3lzdGlzIChIdWlzbWFuIGV0IGFsLiwgMjAxOCksIGFzIHdlbGwgYXMgb3h5Z2VuIGRlcGxldGlvbiBhbmQgaGFiaXRhdCBkZWdyYWRhdGlvbiBpbiBsYWtlcyAoQXJlbmQgZXQgYWwuLCAyMDExKS4gQmVjYXVzZSBsYWtlcyBwcm92aWRlIGEgcmFuZ2Ugb2YgY3J1Y2lhbCBlY29zeXN0ZW0gc2VydmljZXMgKFdpbHNvbiAmIENhcnBlbnRlciwgMTk5OSksIHRoZXJlIGlzIGEgZ3Jvd2luZyBuZWVkIGZvciBtYW5hZ2VtZW50IGFwcHJvYWNoZXMgdGhhdCByZXN0b3JlIHdhdGVyIHF1YWxpdHkgaW4gZnJlc2h3YXRlciBzeXN0ZW1zLiBXaXRoaW4gdGhlIGZyYW1ld29yayBvZiBVbml0ZWQgTmF0aW9ucyBTdXN0YWluYWJsZSBEZXZlbG9wLW1lbnQgR29hbCA2IChDbGVhbiBXYXRlciBhbmQgU2FuaXRhdGlvbiksIHNldmVyYWwgZGlzdGluY3QgdGFyZ2V0cyByZWxhdGUgdG8gdGhlIGltcHJvdmVtZW50IG9mIHdhdGVyIHF1YWxpdHkgYW5kIHByb3RlY3Rpb24gb2YgYXF1YXRpYyBlY29zeXN0ZW1zIGJ5IDIwMzAgKFVuaXRlZCBOYXRpb25zLCAyMDE1KS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyOC4yOTg4NjQzNjQ2MjQwMjMsImxlZnQiOjguNTQ5MjA1NzgwMDI5Mjk3LCJ3aWR0aCI6MzkuMzcxNDk0MjkzMjEyODksImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MjkuOTkxMTM0NjQzNTU0Njg4LCJsZWZ0Ijo4LjU0OTIwNTc4MDAyOTI5Nywid2lkdGgiOjM5LjM2NTExOTkzNDA4MjAzLCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMxLjY4MzQwNjgyOTgzMzk4NCwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNjk2NjMyMzg1MjUzOSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozMy4zNjc5NzMzMjc2MzY3MiwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNzU2NzkwMTYxMTMyOCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNS4wNjAyNDkzMjg2MTMyOCwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNzA5NjQwNTAyOTI5NywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNi43NTI1MTc3MDAxOTUzMSwibGVmdCI6OC41NDkyMDg2NDEwNTIyNDYsIndpZHRoIjozOS4zNzY5NjA3NTQzOTQ1MywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC40MzcwODgwMTI2OTUzMSwibGVmdCI6OC41NDkyMDg2NDEwNTIyNDYsIndpZHRoIjozOS4zNjY1Njk1MTkwNDI5NywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0MC4xMjkzNTYzODQyNzczNDQsImxlZnQiOjguNTQ5MjA4NjQxMDUyMjQ2LCJ3aWR0aCI6MzkuMzY0Mzc5ODgyODEyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0MS44MjE2MjQ3NTU4NTkzNzUsImxlZnQiOjguNTQ5MjA4NjQxMDUyMjQ2LCJ3aWR0aCI6MzkuMzY1NjczMDY1MTg1NTUsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDMuNTA2MTk1MDY4MzU5Mzc1LCJsZWZ0Ijo4LjU0OTM4OTgzOTE3MjM2Mywid2lkdGgiOjM5LjM3MDM5MTg0NTcwMzEyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0NS4xOTg0NjM0Mzk5NDE0MDYsImxlZnQiOjguNTQ5NTcyOTQ0NjQxMTEzLCJ3aWR0aCI6MzkuMzY1ODQ0NzI2NTYyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ni44OTA3MzU2MjYyMjA3LCJsZWZ0Ijo4LjU0OTU3Mjk0NDY0MTExMywid2lkdGgiOjM5LjM3Mjk1NTMyMjI2NTYyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0OC41NzUzMDIxMjQwMjM0NCwibGVmdCI6OC41NDk1NzI5NDQ2NDExMTMsIndpZHRoIjozOS4zNzI0MjEyNjQ2NDg0NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1MC4yNjc1NzgxMjUsImxlZnQiOjguNTQ5NTc2NzU5MzM4Mzc5LCJ3aWR0aCI6MzkuMzcxMTI0MjY3NTc4MTI1LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjUxLjk1OTg0MjY4MTg4NDc2NiwibGVmdCI6OC41NDk1NzY3NTkzMzgzNzksIndpZHRoIjozOS4zNjc4MzU5OTg1MzUxNTYsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTMuNTc0MTE5NTY3ODcxMDk0LCJsZWZ0Ijo4LjU0OTU3Njc1OTMzODM3OSwid2lkdGgiOjM5LjM3MDc3NzEzMDEyNjk1LCJoZWlnaHQiOjEuMzM2ODk1NTg1MDYwMTE5Nn19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjU1LjI2NjM5MTc1NDE1MDM5LCJsZWZ0Ijo4LjU0OTc2MDgxODQ4MTQ0NSwid2lkdGgiOjM5LjM2NjU4MDk2MzEzNDc2NiwiaGVpZ2h0IjoxLjMzNjg5NTU4NTA2MDExOTZ9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1Ny4wMjg5NTM1NTIyNDYwOTQsImxlZnQiOjguNTQ5OTQ0ODc3NjI0NTEyLCJ3aWR0aCI6MzkuMzYyMDI2MjE0NTk5NjEsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTguNzEzNTIzODY0NzQ2MDk0LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjkuMjk5OTgyMDcwOTIyODUyLCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjYwLjQwNTc5MjIzNjMyODEyNSwibGVmdCI6MTAuNjIyNDU1NTk2OTIzODI4LCJ3aWR0aCI6MzcuMjg3ODY4NDk5NzU1ODYsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NjIuMDk4MDYwNjA3OTEwMTU2LCJsZWZ0Ijo4LjU0OTk0MzkyMzk1MDE5NSwid2lkdGgiOjM5LjM3MzMyNTM0NzkwMDM5LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjYzLjc4MjYzMDkyMDQxMDE1NiwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzMxMjY5ODM2NDI1OCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2NS40NzQ5MDY5MjEzODY3MiwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNjUxMTk5MzQwODIwMywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2Ny4xNjcxNzUyOTI5Njg3NSwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNjk0ODAxMzMwNTY2NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2OC43ODE0NDgzNjQyNTc4MSwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzczMzg0MDk0MjM4MywiaGVpZ2h0IjoxLjMzNjg5MTUzMTk0NDI3NX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjcwLjU0NDAxMzk3NzA1MDc4LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjM5LjM2OTMwODQ3MTY3OTY5LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjcyLjIzNjI4OTk3ODAyNzM0LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjM5LjM3MjAzMjE2NTUyNzM0NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo3My45MjA4NDUwMzE3MzgyOCwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzEzMDczNzMwNDY4NzUsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNDM0NCJ9fV0%3D. Urgent investment projected at ₹50,000 crore nationally is essential to sever this pollution pipeline, safeguarding ecosystems and communities.

6.6. Climate Change and Rising Temperatures: Catalysts for Algal Growth

Climate change acts as a potent amplifier of eutrophication in India's lakes, transforming marginal nutrient excesses into rampant harmful algal blooms through rising temperatures, prolonged stratification, and disrupted monsoon regimes. Global warming has elevated average air temperatures across India by 0.7°C since the 1970s, with lake surface waters warming at rates up to 0.5°C per decade, extending the thermal window for algal proliferation (Sitote & Gebremedhine, 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllhYnNyYSBNZWxhayIsImZhbWlseSI6IlNpdG90ZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNaWNoYWVsIEdpcmltYXkiLCJmYW1pbHkiOiJHZWJyZW1lZGhpbmUifX1dLCJpZHMiOnsiaXNzbiI6W10sImlzYm4iOltdLCJkb2kiOiIxMC4yMDk0NC9wcmVwcmludHMyMDI0MDkuMDcwNC52MSIsIm9wZW5hbGV4IjoiVzQ0MDI0NjU0MTQiLCJleGEiOiJodHRwczovL3d3dy5wcmVwcmludHMub3JnL21hbnVzY3JpcHQvMjAyNDA5LjA3MDQifSwidGl0bGUiOiJDb21wcmVoZW5zaXZlIFJldmlldyBvZiBFdXRyb3BoaWNhdGlvbiBpbiBGcmVzaHdhdGVyIEVjb3N5c3RlbXM6IENhdXNlcywgRWZmZWN0cywgQXNzZXNzbWVudCwgYW5kIE1hbmFnZW1lbnQgU3RyYXRlZ2llcyIsImFic3RyYWN0IjoiVGhpcyByZXZpZXcgZXhhbWluZXMgZXV0cm9waGljYXRpb24gaW4gZnJlc2h3YXRlciBlY29zeXN0ZW1zLCBhIHdpZGVzcHJlYWQgZW52aXJvbm1lbnRhbCBpc3N1ZSBzdGVtbWluZyBmcm9tIGV4Y2Vzc2l2ZSBudXRyaWVudCBpbnB1dCwgbGFyZ2VseSBkdWUgdG8gaHVtYW4gYWN0aXZpdGllcy4gVGhlIHN0dWR5IGV4cGxvcmVzIHRoZSBvcmlnaW5zLCBjb25zZXF1ZW5jZXMsIGV2YWx1YXRpb24gdGVjaG5pcXVlcywgYW5kIGNvbnRyb2wgbWV0aG9kcyBmb3IgZXV0cm9waGljYXRpb24uIFByaW1hcnkgY2F1c2VzIGluY2x1ZGUgYWdyaWN1bHR1cmFsIGRpc2NoYXJnZSwgdXJiYW4gZGV2ZWxvcG1lbnQsIGFuZCBjbGltYXRlIHNoaWZ0cy4gVGhlIGVmZmVjdHMgYXJlIGZhci1yZWFjaGluZywgaW1wYWN0aW5nIGJpb2RpdmVyc2l0eSwgd2F0ZXIgcXVhbGl0eSwgYW5kIHNvY2lvLWVjb25vbWljIGZhY3RvcnMuIFRoZSBhc3Nlc3NtZW50IHV0aWxpemVzIGEgY29tYmluYXRpb24gb2YgY2hlbWljYWwsIHBoeXNpY2FsLCBhbmQgYmlvbG9naWNhbCBtYXJrZXJzLCB3aXRoIGN1dHRpbmctZWRnZSB0ZWNobm9sb2dpZXMgbGlrZSByZW1vdGUgc2Vuc2luZyBpbXByb3ZpbmcgbW9uaXRvcmluZyBlZmZvcnRzLiBNaXRpZ2F0aW9uIHN0cmF0ZWdpZXMgYXJlIGRpdmVyc2UsIGluY29ycG9yYXRpbmcgcHJldmVudGF0aXZlIGFjdGlvbnMgKHN1Y2ggYXMgbnV0cmllbnQgbGltaXRhdGlvbiBhbmQgbGFuZCBtYW5hZ2VtZW50KSwgY29ycmVjdGl2ZSBtZWFzdXJlcyAoaW5jbHVkaW5nIGJpby1tYW5pcHVsYXRpb24gYW5kIGNoZW1pY2FsIGFwcGxpY2F0aW9ucyksIGFuZCByZWd1bGF0b3J5IGluaXRpYXRpdmVzIChsaWtlIHdhdGVyIHF1YWxpdHkgZ3VpZGVsaW5lcyBhbmQgbnV0cmllbnQgZXhjaGFuZ2Ugc2NoZW1lcykuIFdoaWxlIGNhc2Ugc3R1ZGllcyBkZW1vbnN0cmF0ZSBzdWNjZXNzZnVsIGludGVydmVudGlvbnMsIHRoZXkgYWxzbyByZXZlYWwgb25nb2luZyBjaGFsbGVuZ2VzLCBwYXJ0aWN1bGFybHkgZWNvc3lzdGVtIHVucHJlZGljdGFiaWxpdHkgYW5kIGNsaW1hdGUgY2hhbmdlIGluZmx1ZW5jZXMuIFRoZSByZXNlYXJjaCBoaWdobGlnaHRzIHRoZSBwcm9taXNlIG9mIGVtZXJnaW5nIHRlY2hub2xvZ2llcywgbm90YWJseSBtYWNoaW5lIGxlYXJuaW5nLCBpbiBlbmhhbmNpbmcgbWFuYWdlbWVudCBwcmFjdGljZXMuIEtub3dsZWRnZSBnYXBzIHBlcnNpc3QsIGVzcGVjaWFsbHkgcmVnYXJkaW5nIGV1dHJvcGhpY2F0aW9uLWJpb2RpdmVyc2l0eSBkeW5hbWljcyBpbiB0cm9waWNhbCBlbnZpcm9ubWVudHMgYW5kIHRoZSBzb2Npby1lY29ub21pYyByYW1pZmljYXRpb25zIG9mIGludGVydmVudGlvbnMuIFRoZSBzdHVkeSBlbXBoYXNpemVzIHRoZSBpbXBvcnRhbmNlIG9mIGZsZXhpYmxlLCBjb21wcmVoZW5zaXZlIGFwcHJvYWNoZXMgdGhhdCBibGVuZCBzY2llbnRpZmljIGluc2lnaHRzIHdpdGggcHJhY3RpY2FsIHNvbHV0aW9ucyBhbmQgc3Ryb25nIHBvbGljeSBzdHJ1Y3R1cmVzIHRvIHRhY2tsZSB0aGlzIGludHJpY2F0ZSBlbnZpcm9ubWVudGFsIGNoYWxsZW5nZSBpbiBhbiBlcmEgb2YgY2xpbWF0ZSBjaGFuZ2UuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjIwOTQ0L3ByZXByaW50czIwMjQwOS4wNzA0LnYxIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjA5NDQvcHJlcHJpbnRzMjAyNDA5LjA3MDQudjEiLCJpc3N1ZWREYXRlIjoiMjAyNC0wOS0xMCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTEiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IlByZXByaW50cy5vcmciLCJpbXBhY3RGYWN0b3IiOiIxLjE1In0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjowLCJjaXRlZEJ5Q291bnQiOjR9fSwicXVvdGUiOiJUaGUgc3R1ZHkgZW1waGFzaXplcyB0aGUgaW1wb3J0YW5jZSBvZiBmbGV4aWJsZSwgY29tcHJlaGVuc2l2ZSBhcHByb2FjaGVzIHRoYXQgYmxlbmQgc2NpZW50aWZpYyBpbnNpZ2h0cyB3aXRoIHByYWN0aWNhbCBzb2x1dGlvbnMgYW5kIHN0cm9uZyBwb2xpY3kgc3RydWN0dXJlcyB0byB0YWNrbGUgdGhpcyBpbnRyaWNhdGUgZW52aXJvbm1lbnRhbCBjaGFsbGVuZ2UgaW4gYW4gZXJhIG9mIGNsaW1hdGUgY2hhbmdlLiBFdXRyb3BoaWNhdGlvbiwgY2F1c2VkIGJ5IGV4Y2Vzc2l2ZSBudXRyaWVudCBlbnJpY2htZW50IGluIHdhdGVyIGJvZGllcywgaXMgYSBzaWduaWZpY2FudCBlbnZpcm9ubWVudGFsIGNoYWxsZW5nZSBmb3IgZnJlc2h3YXRlciBlY29zeXN0ZW1zIG9uIGEgZ2xvYmFsIHNjYWxlLiBQcmltYXJpbHkgZHJpdmVuIGJ5IG5pdHJvZ2VuIGFuZCBwaG9zcGhvcnVzIGlucHV0cywgdGhpcyBwcm9jZXNzIHJlc3VsdHMgaW4gdGhlIG92ZXJwcm9kdWN0aW9uIG9mIGFsZ2FlIGFuZCBhcXVhdGljIHBsYW50cywgbGVhZGluZyB0byBhIGNhc2NhZGUgb2YgZWNvbG9naWNhbCBkaXN0dXJiYW5jZXMsIGluY2x1ZGluZyBveHlnZW4gZGVwbGV0aW9uIChoeXBveGlhKSwgbG9zcyBvZiBiaW9kaXZlcnNpdHksIGFuZCBzZXZlcmUgd2F0ZXIgcXVhbGl0eSBkZWdyYWRhdGlvbiAoS2FrYWRlIGV0IGFsLiwgMjAyMTsgTWVlcmhvZmYgZXQgYWwuLCAyMDIyOyBXdXJ0c2JhdWdoIGV0IGFsLiwgMjAxOSkuIn1d. Cyanobacteria, such as Microcystis and Dolichospermum, thrive optimally between 25-35°C, temperatures increasingly common during extended summers that desynchronize with traditional cooler periods (Nõges et al., 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRpaW5hIiwiZmFtaWx5IjoiTsO1Z2VzIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hc2ltZSIsImZhbWlseSI6IkphbmF0aWFuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlJlZXQiLCJmYW1pbHkiOiJMYXVnYXN0ZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQZWV0ZXIiLCJmYW1pbHkiOiJOw7VnZXMifX1dLCJpZHMiOnsiaXNzbiI6WyIyMzUxLTk4OTQiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMTYvai5nZWNjby4yMDIwLmUwMTM2OSIsIm1hZyI6IjMxMDU0NDYwOTAiLCJvcGVuYWxleCI6IlczMTA1NDQ2MDkwIiwiZXhhIjoiaHR0cHM6Ly93d3cuc2NpZW5jZWRpcmVjdC5jb20vc2NpZW5jZS9hcnRpY2xlL3BpaS9TMjM1MTk4OTQyMDMwOTEwMCJ9LCJ0aXRsZSI6IlBvc3Qtc292aWV0IGNoYW5nZXMgaW4gbml0cm9nZW4gYW5kIHBob3NwaG9ydXMgc3RvaWNoaW9tZXRyeSBpbiB0d28gbGFyZ2Ugbm9uLXN0cmF0aWZpZWQgbGFrZXMgYW5kIHRoZSBpbXBhY3Qgb24gcGh5dG9wbGFua3RvbiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2ouZ2VjY28uMjAyMC5lMDEzNjkiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2ouZ2VjY28uMjAyMC5lMDEzNjkiLCJpc3N1ZWREYXRlIjoiMjAyMC0xMS0yMCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTEiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6Ikdsb2JhbCBFY29sb2d5IGFuZCBDb25zZXJ2YXRpb24iLCJ2b2x1bWUiOjI0LCJpbXBhY3RGYWN0b3IiOiIzLjU5In0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkVsc2V2aWVyIEJWIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo3MSwiY2l0ZWRCeUNvdW50IjoxNH19LCJxdW90ZSI6IlJhbmRvbSBGb3Jlc3QgYW5hbHlzaXMgaW5kaWNhdGVkIGFuIGltcG9ydGFudCByb2xlIG9mIGxpZ2h0IGxpbWl0YXRpb24gaW4gYm90aCBsYWtlcy4gQ29uc3RhbnRseSBsb3dlciBsZXZlbHMgb2YgTiBpbiB0aGUgZGVlcGVyIExha2UgUGVpcHNpIGZhdm91cmVkIE4yLWZpeGluZyBzcGVjaWVzLCB3aGljaCwgYXMgYSBwYXJhZG94LCBiZWNhbWUgUC1saW1pdGVkLiBBcyBjbGltYXRlIHdhcm1pbmcgcmVpbmZvcmNlcyBldXRyb3BoaWNhdGlvbiBwaGVub21lbmEgaW4gbGFrZXMgYnkgaW5jcmVhc2luZyBpbnRlcm5hbCBudXRyaWVudCBsb2FkaW5nIGFuZCBmYXZvdXJpbmcgYmxvb20tZm9ybWluZyBjeWFub2JhY3RlcmlhLCBtb3JlIHN0cmluZ2VudCBtZWFzdXJlcyB3b3VsZCBiZSBuZWVkZWQgdG8gZnVydGhlciBsaW1pdCBudXRyaWVudCBsb2FkcyAoZXNwZWNpYWxseSB0aGF0IG9mIFApIHRvIGxha2VzIHRocm91Z2ggaW1wcm92ZWQgd2FzdGV3YXRlciB0cmVhdG1lbnQgYW5kIGluY3JlYXNlZCBlZmZpY2llbmN5IG9mIGZlcnRpbGlzZXIgYXBwbGljYXRpb24uIExvbmctdGVybSBjaGFuZ2VzIGluIG51dHJpZW50IGxvYWRpbmcgYW5kIHN0b2ljaGlvbWV0cnkgaW4gd2F0ZXIgYm9kaWVzIHJlZmxlY3QgY2hhbmdlcyBpbiBhZ3JpY3VsdHVyYWwgcHJhY3RpY2VzIGFuZCB3YXN0ZXdhdGVyIHRyZWF0bWVudC4ifV0%3D. This thermal boost enhances metabolic rates, boosting photosynthetic efficiency by 10-20% per °C rise, while suppressing competitors like diatoms that favor cooler conditions. Concurrently, erratic monsoons intensified by 20-30% in extremes yet reduced in reliability alter nutrient dynamics: heavy deluges mobilize legacy phosphorus from agricultural soils into lakes, while prolonged dry spells concentrate effluents in shrinking water volumes (Kumar & Mahajan, 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlBhd2FuIiwiZmFtaWx5IjoiS3VtYXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW1icmlzaCBLdW1hciIsImZhbWlseSI6Ik1haGFqYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyNTIzLTM5NjMiLCIyNTIzLTM5NzEiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMDcvczQyNDUyLTAyMC0zMDgyLTgiLCJtYWciOiIzMDM1OTYwMjI4Iiwib3BlbmFsZXgiOiJXMzAzNTk2MDIyOCJ9LCJ0aXRsZSI6IlRyb3BoaWMgc3RhdHVzIGFuZCBpdHMgcmVndWxhdGluZyBmYWN0b3IgZGV0ZXJtaW5hdGlvbiBhdCB0aGUgUmV3YWxzYXIgTGFrZSwgbm9ydGh3ZXN0IEhpbWFsYXlhIChIUCksIEluZGlhLCBiYXNlZCBvbiBzZWxlY3RlZCBwYXJhbWV0ZXJzIGFuZCBtdWx0aXZhcmlhdGUgc3RhdGlzdGljYWwgYW5hbHlzaXMiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAwNy9zNDI0NTItMDIwLTMwODItOCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9saW5rLnNwcmluZ2VyLmNvbS9jb250ZW50L3BkZi8xMC4xMDA3L3M0MjQ1Mi0wMjAtMzA4Mi04LnBkZiIsImlzc3VlZERhdGUiOiIyMDIwLTA2LTIyIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiU04gQXBwbGllZCBTY2llbmNlcyIsImlzc3VlIjo3LCJ2b2x1bWUiOjIsImltcGFjdEZhY3RvciI6IjcuMzUifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiU3ByaW5nZXIgTmF0dXJlIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo0MCwiY2l0ZWRCeUNvdW50IjoxOX19LCJxdW90ZSI6IkNoYW5naW5nIHF1YWxpdHkgaGFzIGNyZWF0ZWQgdGhlIHdhdGVyIHNjYXJjaXR5IGFuZCAgYSBodWdlIG1hc3Mgb2YgcG9wdWxhdGlvbiB0aHVzIGJlZW4gZGVwcml2ZWQgZnJvbSB1c2luZyAgdGhlIGZyZXNod2F0ZXIgZm9yIHZhcmlvdXMgcHVycG9zZXMgWzMsIDRdLiBIb3dldmVyLCB0aGUgIG9ibm94aW91cyBtYXRlcmlhbHMgdGhhdCBzdGF5IGluIHdhdGVyIGJvZGllcyBmb3IgYSBsb25nLXRpbWUgcGVyaW9kIGJyaW5nIGNoYW5nZXMgaW4gdGhlIHBoeXNpb2NoZW1pY2FsIGNoYXItYWN0ZXJpc3RpY3MgYXMgd2VsbCBhcyB0aGUgbmF0dXJhbCBzY2VuYXJpbyBvZiB0aGUgbGFrZXMgWzVdLiAgVGhlIEhpbWFsYXlhbiBsYWtlcyBhcmUgdW5kZXIgc3RyZXNzIGR1ZSB0byBtaXNtYW5hZ2VkIGh1bWFuIGFjdGl2aXRpZXMsIHdoaWNoIGluY3JlYXNlIG51dHJpZW50cyBhbmQgcG9sbHV0aW9uIGluIHRlcm1zIG9mIGhlYXZ5IG1ldGFscyB0b3hpY2l0eSBhbmQgYmFjdGVyaWFsIGNvbnRhbWluYXRpb24gIFs24oCTOF0uIFRoZSBtYW5hZ2VtZW50IGFuZCBwcmVzZXJ2YXRpb24gb2YgdGhlIGxha2VzIGF0ICBwcmVzZW50IGhhdmUgYmVjb21lIGEgZ3JlYXQgY2hhbGxlbmdlIGZvciBlbnZpcm9ubWVudGFsLWlzdHMgYW5kIG1hbmFnZXJzIFs1LCA5XS4gVGhlIGV1dHJvcGhpY2F0aW9uIGlzIGFsc28gYSBtYWluIHByb2JsZW0gb2YgdGhlICBmcmVzaHdhdGVyIGxha2VzLCBhbmQgaXQgaXMgc3BlY2lmaWVkIGZvciB0aGUgbGFrZXMgaGF2aW5nICBudXRyaWVudCBlbnJpY2htZW50LiBJdCBjYXVzZXMgZXhjZXNzaXZlIGdyb3d0aCBvZiBhbGdhZSAgYW5kIGlzIGNvbnNpZGVyZWQgdG8gYmUgcmVzcG9uc2libGUgZm9yIGRlZ3JhZGluZyB0aGUgIHF1YWxpdGF0aXZlIG5hdHVyZSBvZiB0aGUgbGFrZXMgWzUsIDEwXS4gVGhlIGV1dHJvcGhpY2F0aW9uICBpcyBmb2xsb3dlZCBieSB0aGUgYWxnYWwgYmxvb20sIHdoaWNoIGdlbmVyYXRlcyB0b3hpbiwgIHdvcnNlbnMgd2F0ZXIgcXVhbGl0eSwgZGVwbGV0ZXMgZGlzc29sdmVkIG94eWdlbiAoRE8pICogUGF3YW4gS3VtYXIsIHBhd2FuLmV2c0BnbWFpbC5jb20gfCAxRGVwYXJ0bWVudCBvZiBFbnZpcm9ubWVudGFsIFNjaWVuY2VzLCBDZW50cmFsIFVuaXZlcnNpdHkgb2YgSGltYWNoYWwgUHJhZGVzaCwgRGhhcmFtc2hhbGEgMTc2MjE1LCBJbmRpYS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2OC41Nzg0NjA2OTMzNTkzOCwibGVmdCI6OC41NzE0MTg3NjIyMDcwMzEsIndpZHRoIjo0MC4zMjc1MTQ2NDg0Mzc1LCJoZWlnaHQiOjEuNTE5ODU1MjYwODQ4OTk5fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NzAuMTU4OTk2NTgyMDMxMjUsImxlZnQiOjguNTY5NzM4Mzg4MDYxNTIzLCJ3aWR0aCI6NDAuMzU0NzI0ODg0MDMzMiwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjcxLjczOTU0NzcyOTQ5MjE5LCJsZWZ0Ijo4LjU2OTczODM4ODA2MTUyMywid2lkdGgiOjQwLjM0OTY5MzI5ODMzOTg0NCwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjczLjMyMDA5MTI0NzU1ODYsImxlZnQiOjguNTY4MDU5OTIxMjY0NjQ4LCJ3aWR0aCI6NDAuMDIwMDg4MTk1ODAwNzgsImhlaWdodCI6MS41MTk4NTUyNjA4NDg5OTl9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo3NC45MDA2NDIzOTUwMTk1MywibGVmdCI6OC41NjgwNTk5MjEyNjQ2NDgsIndpZHRoIjozOS45OTk5Mjc1MjA3NTE5NSwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjc2LjQ4MTE4NTkxMzA4NTk0LCJsZWZ0Ijo4LjU2ODA1NzA2MDI0MTcsIndpZHRoIjo0MC4zNTcwNzQ3Mzc1NDg4MywiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjc4LjA2MTcyOTQzMTE1MjM0LCJsZWZ0Ijo4LjU2OTczNjQ4MDcxMjg5LCJ3aWR0aCI6NDAuMzUzNzEzOTg5MjU3ODEsImhlaWdodCI6MS41MTk4NTUyNjA4NDg5OTl9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2MC42NzU3Mjc4NDQyMzgyOCwibGVmdCI6NTEuNDI3MTY1OTg1MTA3NDIsIndpZHRoIjo0MC4zNzg1NjY3NDE5NDMzNiwiaGVpZ2h0IjoxOC45MDU4NTUxNzg4MzMwMDh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo4MS45MjczMzc2NDY0ODQzOCwibGVmdCI6OC40MDE0OTc4NDA4ODEzNDgsIndpZHRoIjo3My45NTQ2OTY2NTUyNzM0NCwiaGVpZ2h0IjoyLjY3ODc2MjY3NDMzMTY2NX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEifX1d. In peri-urban lakes like Bellandur and Hussainsagar, these shifts have synchronized with post-2010 bloom escalations, underscoring climate's role in tipping ecosystems toward cyanobacterial dominance (Gobler et al., 2024; Nair & Nayak, 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkNocmlzdG9waGVyIEouIiwiZmFtaWx5IjoiR29ibGVyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlJ1dGggVy4iLCJmYW1pbHkiOiJEcmlua3dhdGVyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkEuIFcuIiwiZmFtaWx5IjoiQW50aG9ueSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJKZW5uaWZlciBBLiIsImZhbWlseSI6IkdvbGVza2kifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW5uIE1hcmllIEUuIiwiZmFtaWx5IjoiRmFtdWxhcm8tUGVjb3JhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1pY2hhZWwgQi4iLCJmYW1pbHkiOiJXYWxsYWNlIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5vcmEgUi4gVy4iLCJmYW1pbHkiOiJTdHJhcXVhZGluZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJSb25vam95IiwiZmFtaWx5IjoiSGVtIn19XSwiaWRzIjp7Imlzc24iOlsiMTY2NC0zMDJYIl0sImlzYm4iOltdLCJkb2kiOiIxMC4zMzg5L2ZtaWNiLjIwMjQuMTQ2NDY4NiIsInBtaWQiOiIzOTU2ODk5NyIsIm9wZW5hbGV4IjoiVzQ0MDQxMjA5NTIiLCJleGEiOiJodHRwczovL3d3dy5mcm9udGllcnNpbi5vcmcvam91cm5hbHMvbWljcm9iaW9sb2d5L2FydGljbGVzLzEwLjMzODkvZm1pY2IuMjAyNC4xNDY0Njg2L2Z1bGwifSwidGl0bGUiOiJTZXdhZ2UtYW5kIGZlcnRpbGl6ZXItZGVyaXZlZCBudXRyaWVudHMgYWx0ZXIgdGhlIGludGVuc2l0eSwgZGl2ZXJzaXR5LCBhbmQgdG94aWNpdHkgb2YgaGFybWZ1bCBjeWFub2JhY3RlcmlhbCBibG9vbXMgaW4gZXV0cm9waGljIGxha2VzIiwiYWJzdHJhY3QiOiJDeWFub2JhY3RlcmlhbCBoYXJtZnVsIGFsZ2FsIGJsb29tcyAoQ0hBQnMpIGFyZSBwcm9tb3RlZCBieSBleGNlc3NpdmUgbnV0cmllbnQgbG9hZGluZyBhbmQsIHdoaWxlIGZlcnRpbGl6ZXJzIGFuZCBzZXdhZ2UgYXJlIHRoZSBtb3N0IHByZXZhbGVudCBleHRlcm5hbCBudXRyaWVudCBzb3VyY2VzIGluIG1vc3Qgd2F0ZXJzaGVkcywgdGhlIGRpZmZlcmVudGlhbCBlZmZlY3RzIG9mIHRoZXNlIG51dHJpZW50IHNvdXJjZXMgb24gQ0hBQnMgYXJlIHVua25vd24uIEhlcmUsIHdlIHRyYWNrZWQgQ0hBQnMgYW5kIHBlcmZvcm1lZCBleHBlcmltZW50cyBpbiBmaXZlIGRpc3RpbmN0IGxha2VzIGFjcm9zcyB0aGUgTm9ydGhlcm4gVVMgaW5jbHVkaW5nIExha2UgRXJpZS4gRmVydGlsaXplcnMgd2l0aCBhbW1vbml1bSBhbmQgb3J0aG9waG9zcGhhdGUsIG1lbWJyYW5lICgwLjIgzrxtKS1maWx0ZXJlZCBzZXdhZ2UgKGRvbWluYXRlZCBieSByZWR1Y2VkIGZvcm1zIG9mIG5pdHJvZ2VuKSBzYW5kLWFuZCBtZW1icmFuZS1maWx0ZXJlZCBzZXdhZ2UgKGRvbWluYXRlZCBieSBuaXRyYXRlKSwgYW5kIGFuIGlub3JnYW5pYyBudXRyaWVudCBzb2x1dGlvbiBvZiBhbW1vbml1bSBhbmQgb3J0aG9waG9zcGhhdGUgd2VyZSB1c2VkIGFzIGV4cGVyaW1lbnRhbCBudXRyaWVudCBzb3VyY2VzIGZvciBDSEFCcyBhdCBOLWVxdWl2YWxlbnQsIGVudmlyb25tZW50YWxseSByZWFsaXN0aWMgY29uY2VudHJhdGlvbnMuIFBoeXRvcGxhbmt0b24gY29tbXVuaXRpZXMgd2VyZSBldmFsdWF0ZWQgZmx1b3JvbWV0cmljYWxseSwgbWljcm9zY29waWNhbGx5LCBhbmQgdmlhIGhpZ2ggdGhyb3VnaHB1dCBzZXF1ZW5jaW5nIG9mIHRoZSAxNlMgclJOQSBnZW5lLCBhbmQgbGV2ZWxzIG9mIG1pY3JvY3lzdGluIGFuZCB0aGUgzrQgMTUgTiBjb250ZW50IG9mIHBhcnRpY3VsYXRlIG9yZ2FuaWMgbml0cm9nZW4gKM60UE8gMTUgTikgd2VyZSBxdWFudGlmaWVkLiBGZXJ0aWxpemVyIGFuZCBib3RoIHNvdXJjZXMgb2Ygd2FzdGV3YXRlciBpbmNyZWFzZWQgdGhlIGFidW5kYW5jZSBvZiBjeWFub2JhY3RlcmlhIGluIGFsbCBleHBlcmltZW50cyBhY3Jvc3MgYWxsIGZpdmUgbGFrZXMgKCBwICZhbXA7bHQ7IDAuMDUgZm9yIGFsbCkgd2hlcmVhcyBlZmZlY3RzIG9uIGV1a2FyeW90aWMgcGh5dG9wbGFua3RvbiB3ZXJlIGxpbWl0ZWQuIFNhbmQtZmlsdGVyZWQgc2V3YWdlIGNvbnRhaW5lZCBsZXNzIFAsIG9yZ2FuaWMgbWF0dGVyLCBhbmQgYW1tb25pdW0gYnV0IG1vcmUgbml0cmF0ZSBhbmQgaGFkIGEgMjUlIGxlc3MgcG90ZW50IHN0aW11bGF0b3J5IGVmZmVjdCBvbiBjeWFub2JhY3RlcmlhIHRoYW4gbWVtYnJhbmUtZmlsdGVyZWQgc2V3YWdlLCBzdWdnZXN0aW5nIG5pdHJpZmljYXRpb24gbWF5IHBsYXkgYSByb2xlIGluIHJlZHVjaW5nIENIQUJzLiBGZXJ0aWxpemVyIGluY3JlYXNlZCBtaWNyb2N5c3RpbiBsZXZlbHMgYW5kIGRlY3JlYXNlZCB0aGUgzrRQTyAxNSBOIHdoZXJlYXMgd2FzdGV3YXRlciBpbmNyZWFzZWQgzrRQTyAxNSBOICggcCAmYW1wO2x0OyAwLjA1IGZvciBhbGwpLiBNaWNyb2N5c3RpcyB3YXMgdGhlIGdlbnVzIG1vc3QgY29uc2lzdGVudGx5IHByb21vdGVkIGJ5IG51dHJpZW50IHNvdXJjZXMgKCBwICZhbXA7bHQ7IDAuMDUgaW4gYWxsIGV4cGVyaW1lbnRzKSwgZm9sbG93ZWQgYnkgQ3lhbm9iaXVtICggcCAmYW1wO2x0OyAwLjA1IGluIDUwJSBvZiBleHBlcmltZW50cyksIHdpdGggaW5jcmVhc2VzIGluIE1pY3JvY3lzdGlzIGJpb21hc3MgY29uc2lzdGVudGx5IGVsaWNpdGVkIGJ5IG1lbWJyYW5lLWZpbHRlcmVkIHdhc3Rld2F0ZXIuIENvbGxlY3RpdmVseSwgcmVzdWx0cyBkZW1vbnN0cmF0ZSB0aGF0IGRpZmZlcmluZyB0eXBlcyBvZiBzZXdhZ2UgZGlzY2hhcmdlIGFuZCBmZXJ0aWxpemVycyBjYW4gcHJvbW90ZSBDSEFCIGludGVuc2l0eSBhbmQgdG94aWNpdHksIHdoaWxlIGNvbmN1cnJlbnRseSBhbHRlcmluZyBDSEFCIGRpdmVyc2l0eSBhbmQgzrRQTyAxNSBOLiBXaGlsZSBtZW1icmFuZS1maWx0ZXJlZCBzZXdhZ2UgY29uc2lzdGVudGx5IGZhdm9yZWQgTWljcm9jeXN0aXMgLCB0aGUgZGlzY2hhcmdlIG9mIHNld2FnZSB0aHJvdWdoIHNhbmRzIG11dGVkIGJsb29tIGludGVuc2l0eSBzdWdnZXN0aW5nIHNhbmQtYmVkcyBtYXkgcmVwcmVzZW50IGEgdG9vbCB0byByZW1vdmUga2V5IG51dHJpZW50cyBhbmQgcGFydGlhbGx5IG1pdGlnYXRlIENIQUJzLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzg5L2ZtaWNiLjIwMjQuMTQ2NDY4NiIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cuZnJvbnRpZXJzaW4ub3JnL2pvdXJuYWxzL21pY3JvYmlvbG9neS9hcnRpY2xlcy8xMC4zMzg5L2ZtaWNiLjIwMjQuMTQ2NDY4Ni9wZGYiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMS0wNiIsImFjY2Vzc2VkRGF0ZSI6IjIwMjYtMDIiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZyb250aWVycyBpbiBNaWNyb2Jpb2xvZ3kiLCJ2b2x1bWUiOjE1LCJpbXBhY3RGYWN0b3IiOiIzLjk5In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTQ2NDY4NiwibGFzdFBhZ2UiOjE0NjQ2ODZ9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRnJvbnRpZXJzIE1lZGlhIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo3MywiY2l0ZWRCeUNvdW50Ijo5fX0sInF1b3RlIjoiZGVyaXZlZCBudXRyaWVudHMgYWx0ZXIgdGhlIGludGVuc2l0eSwgZGl2ZXJzaXR5LCBhbmQgdG94aWNpdHkgb2YgaGFybWZ1bCBjeWFub2JhY3RlcmlhbCBibG9vbXMgLi4uIEN5YW5vYmFjdGVyaWFsIGhhcm1mdWwgYWxnYWwgYmxvb21zIChDSEFCcykgYXJlIHByb21vdGVkIGJ5IGV4Y2Vzc2l2ZSBudXRyaWVudCBsb2FkaW5nIGFuZCwgd2hpbGUgZmVydGlsaXplcnMgYW5kIHNld2FnZSBhcmUgdGhlIG1vc3QgcHJldmFsZW50IGV4dGVybmFsIG51dHJpZW50IHNvdXJjZXMgaW4gbW9zdCB3YXRlcnNoZWRzLCB0aGUgZGlmZmVyZW50aWFsIGVmZmVjdHMgb2YgdGhlc2UgbnV0cmllbnQgLi4uIG9yZ2FuaWMgbml0cm9nZW4gKM60UE8xNU4pIHdlcmUgcXVhbnRpZmllZC4gRmVydGlsaXplciBhbmQgYm90aCBzb3VyY2VzIG9mIHdhc3Rld2F0ZXIgaW5jcmVhc2VkIHRoZSBhYnVuZGFuY2Ugb2YgLi4uIHBoeXRvcGxhbmt0b24gd2VyZSBsaW1pdGVkLiBTYW5kLWZpbHRlcmVkIHNld2FnZSBjb250YWluZWQgbGVzcyBQLCBvcmdhbmljIG1hdHRlciwgYW5kIGFtbW9uaXVtIGJ1dCBtb3JlIG5pdHJhdGUgYW5kIC4uLiBkaWZmZXJpbmcgdHlwZXMgb2Ygc2V3YWdlIGRpc2NoYXJnZSBhbmQgZmVydGlsaXplcnMgY2FuIHByb21vdGUgQ0hBQiBpbnRlbnNpdHkgYW5kIHRveGljaXR5LCB3aGlsZSBjb25jdXJyZW50bHkgLi4uIHNld2FnZSB0aHJvdWdoIHNhbmRzIG11dGVkIGJsb29tIGludGVuc2l0eSBzdWdnZXN0aW5nIHNhbmQtYmVkcyBtYXkgcmVwcmVzZW50IGEgdG9vbCB0byByZW1vdmUga2V5IG51dHJpZW50cyBhbmQgLi4uICggV2F0c29uIGV0IGFsLiwgMjAxNiA7IEJ1bGxlcmphaG4gZXQgYWwuLCAyMDE2IDsgSHVpc21hbiBldCBhbC4sIDIwMTggKSBhbmQgZXhjZXNzaXZlIG51dHJpZW50IGxvYWRpbmcgaXMgdGhlIHByb2Nlc3MgbW9zdCBjb21tb25seSBjaXRlZCBhcyB0aGUgZW52aXJvbm1lbnRhbCBmYWN0b3IgcHJvbW90aW5nIENIQUJzICgifSx7Im9yaWdpbiI6NCwid29yayI6eyJ0eXBlIjo4LCJzdWJ0eXBlIjp7Im9uZW9mS2luZCI6ImFydGljbGVUeXBlIiwiYXJ0aWNsZVR5cGUiOjF9LCJjb250cmlidXRvcnMiOlt7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJOYXZ5YSBWaWtyYW1hbiIsImZhbWlseSI6Ik5haXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUHJhdGVlcCBLdW1hciIsImZhbWlseSI6Ik5heWFrIn19XSwiaWRzIjp7Imlzc24iOlsiMjI5Ni03NzQ1Il0sImlzYm4iOltdLCJkb2kiOiIxMC4zMzg5L2ZtYXJzLjIwMjMuMTA4NzI5NiIsIm9wZW5hbGV4IjoiVzQzODI3MjMxMzMifSwidGl0bGUiOiJVbmNvdmVyaW5nIHdhdGVyIHF1YWxpdHkgYW5kIGV2YWx1YXRpbmcgdnVsbmVyYWJpbGl0aWVzIG9mIHNtYWxsLXNjYWxlIGZpc2hlcmllcyBpbiBDaGlsaWthIExhZ29vbiwgSW5kaWEiLCJhYnN0cmFjdCI6IlNtYWxsLXNjYWxlIGZpc2hlcmllcyAoU1NGcykgcGxheSBhIHN0cm9uZyByb2xlIGluIHN1c3RhaW5pbmcgbWlsbGlvbnMgb2YgbGl2ZWxpaG9vZHMsIGZvb2Qgc2VjdXJpdHksIG51dHJpdGlvbiwgYW5kIGluY29tZSBnbG9iYWxseSBidXQgdGhlIGZpc2hlcnMgZW5nYWdlZCBpbiB0aGlzIHNlY3RvciBzaW11bHRhbmVvdXNseSBleHBlcmllbmNlIGhpZ2ggbGV2ZWxzIG9mIHZ1bG5lcmFiaWxpdHkgYW5kIHByb2Nlc3NlcyBvZiBtYXJnaW5hbGlzYXRpb24uIFNldmVyYWwgZmFjdG9ycyBhcmUgYXR0cmlidXRlZCB0byB0aGUgbXVsdGlkaW1lbnNpb25hbCB2dWxuZXJhYmlsaXRpZXMgdGhlIHNtYWxsLXNjYWxlIGZpc2hlcnMgZXhwZXJpZW5jZSwgc3Bhbm5pbmcgYm90aCBuYXR1cmFsIChlLmcuLCBuYXR1cmFsIGRpc2FzdGVycywgZWNvc3lzdGVtIGNoYW5nZSkgYW5kIGFudGhyb3BvZ2VuaWMgKGUuZy4sIHBvbGljeSBjaGFuZ2UsIGh5ZHJvbG9naWNhbCBpbnRlcnZlbnRpb25zLCBhcXVhY3VsdHVyZSkgcHJlc3N1cmVzLiBXaGlsZSB0aGVyZSBpcyBtdWNoIGxpdGVyYXR1cmUgb24gdmFyaW91cyBuYXR1cmFsIGFuZCBodW1hbiBkcml2ZXJzIG9mIHZ1bG5lcmFiaWxpdHkgaW4gc21hbGwtc2NhbGUgZmlzaGVyeSBjb21tdW5pdGllcywgYW4gYWJzZW5jZSBvZiByZXNlYXJjaCBjb25uZWN0aW5nIHZ1bG5lcmFiaWxpdHkgd2l0aCB3YXRlciBxdWFsaXR5IGlzIGV2aWRlbnQuIEZpc2hlciBjb21tdW5pdGllcyBvZnRlbiB0YWxrIGFib3V0IGZpc2ggaW4gcmVsYXRpb24gdG8gdGhlIGhlYWx0aCBvZiB0aGVpciBhcXVhdGljIGhhYml0YXRzIHdoZXJlaW4gd2F0ZXIgcXVhbGl0eSBpcyBzZWVuIGFzIGEga2V5IHBhcmFtZXRlci4gVGhlIGxpbmsgYmV0d2VlbiBoZWFsdGh5IGZpc2ggYW5kIGdvb2QgcXVhbGl0eSB3YXRlciBoYXMgc2lnbmlmaWNhbnQgaW1wbGljYXRpb25zIGZvciBzdHJvbmcgYW5kIHZpYWJsZSBmaXNoaW5nIGNvbW11bml0aWVzLiBUaGlzIHBhcGVyIGV4YW1pbmVzIHRoZXNlIGxpbmtzIGZ1cnRoZXIgYnkgZm9jdXNpbmcgb24gdGhlIG5hdHVyZSBvZiB2dWxuZXJhYmlsaXRpZXMgY2F1c2VkIGJ5IHdhdGVyIHF1YWxpdHkgY2hhbmdlcyBpbiB0aGUgc21hbGwtc2NhbGUgZmlzaGVyeSBzeXN0ZW0gb2YgQ2hpbGlrYSBMYWdvb24gaW4gSW5kaWEuIFdlIHVuZGVydGFrZSBkZXRhaWxlZCBhbmFseXNpcyBvZiB0aGUgaW52YXNpdmUgc2hyaW1wIGFxdWFjdWx0dXJlIGFjdGl2aXRpZXMgYW5kIGh5ZHJvbG9naWNhbCBpbnRlcnZlbnRpb25zIGZvciBvcGVuaW5nIG9mIGEgbGFnb29uIGlubGV0IHdpdGggdGhlIEJheSBvZiBCZW5nYWwgYXMgdHdvIGRvbWluYW50IGRyaXZlcnMgYWR2ZXJzZWx5IGltcGFjdGluZyB3YXRlciBxdWFsaXR5IGFuZCBpbmNyZWFzaW5nIHZ1bG5lcmFiaWxpdGllcyBvZiB0aGUgZW50aXJlIHNtYWxsLXNjYWxlIGZpc2hlcmllcyBzb2NpYWwtZWNvbG9naWNhbCBzeXN0ZW0uIE91ciBhbmFseXNpcyBzdWdnZXN0cyB0aGF0IHRoZXJlIGFyZSBzdHJvbmcgaW50ZXJjb25uZWN0aW9ucyBiZXR3ZWVuIGNoYW5nZXMgaW4gd2F0ZXIgcXVhbGl0eSBhbmQgdGhlIGxldmVscyBvZiB2dWxuZXJhYmlsaXRpZXMgaW4gdGhlIFNTRnMgb2YgQ2hpbGlrYSBMYWdvb24uIFBvbGx1dGFudHMgc3VjaCBhcyBwZXN0aWNpZGVzLCBhbmQgb3JnYW5pYyBjb21wb3VuZHMgYWNjdW11bGF0ZSBpbiBmaXNoIHRpc3N1ZXMgYW5kIGFmZmVjdCB0aGVpciBncm93dGgsIHJlcHJvZHVjdGlvbiwgYW5kIG92ZXJhbGwgaGVhbHRoLiBUaGlzIGxlZCB0byBkZWNsaW5lcyBpbiBmaXNoIHBvcHVsYXRpb25zLCBtYWtpbmcgaXQgbW9yZSBkaWZmaWN1bHQgZm9yIGZpc2hlcnMgdG8gbWFrZSBhIGxpdmluZy4gSW4gYWRkaXRpb24gdG8gZGlyZWN0IGltcGFjdHMgb24gZmlzaCBwb3B1bGF0aW9ucywgcG9vciB3YXRlciBxdWFsaXR5IGFsc28gaGFzIGluZGlyZWN0IGVmZmVjdHMgb24gdGhlIHNvY2lhbCBhbmQgZWNvbm9taWMgdnVsbmVyYWJpbGl0eSBvZiBTU0ZzLiBGb3IgZXhhbXBsZSwgY29udGFtaW5hdGlvbiBvZiB3YXRlciBzb3VyY2VzIGxlZCB0byB0aGUgcmVkdWNlZCBudW1iZXIgb2YgZmlzaCBzcGVjaWVzIHJlZHVjaW5nIHRoZSBhbW91bnQgb2YgdGltZSBmaXNoZXJzIGNhbiBzcGVuZCBvbiBmaXNoaW5nIGFjdGl2aXRpZXMuIFRoaXMgYWxzbyBhZmZlY3RlZCB0aGUgbWFya2V0YWJpbGl0eSBvZiBmaXNoIHByb2R1Y3RzLCByZWR1Y2VkIGluY29tZSBhbmQgaW5jcmVhc2VkIHBvdmVydHkuIFRvIGZ1bGx5IHVuZGVyc3RhbmQgdGhlIGludGVyY29ubmVjdGlvbnMgYmV0d2VlbiB3YXRlciBxdWFsaXR5IGFuZCB2dWxuZXJhYmlsaXR5IGluIFNTRnMgaW4gQ2hpbGlrYSBMYWdvb24sIGl0IGlzIGltcG9ydGFudCB0byBjb25zaWRlciBib3RoIGVudmlyb25tZW50YWwgYW5kIHNvY2lhbCBmYWN0b3JzLCBhcyB3ZWxsIGFzIHRoZSBjb21wbGV4IGZlZWRiYWNrIGxvb3BzIGJldHdlZW4gdGhlc2UgZmFjdG9ycy4gVGhlIHN0dWR5IGhlbHBzIGluIGJyaWRnaW5nIGEgY3J1Y2lhbCBnYXAgaW4gb3VyIHVuZGVyc3RhbmRpbmcgb2YgdGhlIHJvbGUgb2Ygd2F0ZXIgcXVhbGl0eSBpbiB2dWxuZXJhYmlsaXR5IGFuYWx5c2lzIHdpdGhpbiByZXNvdXJjZSBkZXBlbmRlbnQgY29tbXVuaXRpZXMuIFdlIGNvbmNsdWRlIHdpdGgga2V5IGluc2lnaHRzIG9uIHBvc3NpYmxlIGNvcGluZyByZXNwb25zZXMgYW5kIGFkYXB0aXZlIGNhcGFjaXR5IG5lY2Vzc2FyeSBmb3IgdGhlIHNtYWxsLXNjYWxlIGZpc2hlcmllcyBjb21tdW5pdGllcyB0byB0cmFuc2l0aW9uIHRvd2FyZCB2aWFiaWxpdHkuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjMzODkvZm1hcnMuMjAyMy4xMDg3Mjk2IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5mcm9udGllcnNpbi5vcmcvYXJ0aWNsZXMvMTAuMzM4OS9mbWFycy4yMDIzLjEwODcyOTYvcGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjMtMDYtMjkiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTEwIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJGcm9udGllcnMgaW4gTWFyaW5lIFNjaWVuY2UiLCJ2b2x1bWUiOjEwLCJpbXBhY3RGYWN0b3IiOiI0LjI4In0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkZyb250aWVycyBNZWRpYSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NTEsImNpdGVkQnlDb3VudCI6NX19LCJxdW90ZSI6IldhdGVyIHF1YWxpdHkgb2YgQ2hpbGlrYSBMYWdvb24gaGFzIGRlZ3JhZGVkIGR1ZSB0byBhbnRocm9wb2dlbmljIGludGVydmVudGlvbnMgYW5kIG5hdHVyYWwgZm9yY2VzIG9mIGNoYW5nZS4gVmFyaW91cyBhY3Rpdml0aWVzIGluIHRoZSBDaGlsaWthIExhZ29vbiwgc3VjaCBhcyBpbmR1c3RyaWFsIHdhc3Rld2F0ZXIgZGlzcG9zYWwsIHNld2FnZSBkdW1waW5nLCBhcXVhY3VsdHVyZSwgaHlkcm9sb2dpY2FsIGludGVydmVudGlvbnMsIGFuZCBjeWNsb25lcywgYXJlIHByb2R1Y2luZyBzYWxpbml0eSBjaGFuZ2VzLCBzZWRpbWVudCBkZXBvc2l0aW9uLCBudXRyaWVudCBlbnJpY2htZW50LCBldXRyb3BoaWNhdGlvbiwgYW5kIGRlYWQgem9uZXMsIGFzIHN0YXRlZC4gRmlzaCBzdXJ2aXZhbCBpcyBhdCByaXNrIGFzIGEgcmVzdWx0IG9mIHRoZSBzaWduaWZpY2FudCByZWR1Y3Rpb24gaW4gZW52aXJvbm1lbnRhbCBjaXJjdW1zdGFuY2VzLCB3aGljaCBkaW1pbmlzaGVzIGZpc2hlcm1lbuKAmXMgaW5jb21lIGFuZCBwdXNoZXMgdGhlbSBvdXQgb2Ygc29jaWV0eS4gVGhlc2UgY2F1c2VzIGFmZmVjdGluZyB0aGUgd2F0ZXIgcXVhbGl0eSBvZiB0aGUgTGFnb29uIGRlcGljdCBtYW55IGZhY2VzIG9mIHZ1bG5lcmFiaWxpdHkgaW4gU1NGIGNvbW11bml0aWVzLiBBcyB0aGUgQ2hpbGlrYSBMYWdvb24gZW52aXJvbm1lbnQgaXMgZXhwZWN0ZWQgdG8gdW5kZXJnbyBpbmNyZWFzaW5nbHkgZHJhc3RpYyBjaGFuZ2VzIChzdWNoIGFzIGNhdGNoIGNvbXBvc2l0aW9uLCBjYXRjaCBjYXBhY2l0eSwgYmlvZGl2ZXJzaXR5IHZhcmlhdGlvbnMsIHdhdGVyIHF1YWxpdHksIGFuZCBmaXNoZXJ5IHJldmVudWVzKSwgaW50ZXJuYXRpb25hbCBwb2xpY3kgaGFzIGJlY29tZSBtb3JlIGludGVyZXN0ZWQgaW4gYWRhcHRhdGlvbiBtZWFzdXJlcy4gR2l2ZW4gaG93IGRpdmVyc2UgU1NGIGVjb3N5c3RlbXMgYXJlIGV4cGVjdGVkIHRvIGJlIGJldHdlZW4gY29udGluZW50cywgdGhlcmUgaXMgbm8g4oCcb25lIHNpemUgZml0cyBhbGzigJ0gYXBwcm9hY2guIFRoZSBwYXBlciBvdXRsaW5lZCBzZXZlcmFsIGNvcGluZyBhbmQgYWRhcHRhdGlvbiBtZWNoYW5pc21zIHRvIGltcHJvdmUgd2F0ZXIgcXVhbGl0eSBhbmQgY29udGludWUgZmlzaGluZyBpbiBhIHN1c3RhaW5hYmxlIHdheS4gQWRhcHRhdGlvbiBzdHJhdGVnaWVzIGNhbiBvY2Nhc2lvbmFsbHkgaGF2ZSB1bmZvcmVzZWVuIGNvbnNlcXVlbmNlcyBmb3IgZmlzaGVyaWVzIGFuZCBmaXNoaW5nIGNvbW11bml0aWVzLiBBbnkgdW5leHBlY3RlZCBjb25zZXF1ZW5jZXMgb2YgYWRhcHRhdGlvbiBjYW4gYmUgbWl0aWdhdGVkIHRocm91Z2ggbG9uZy10ZXJtIHBsYW5uaW5nIGFuZCBpZGVudGlmaWNhdGlvbiBvZiBmdXR1cmUgb3B0aW9ucyB0aHJvdWdoIHNjZW5hcmlvIGFuYWx5c2lzLiBUaGUgZmluZGluZ3Mgb2YgdGhpcyBwYXBlciBjYW4gYmUgdXRpbGl6ZWQgdG8gZGV2ZWxvcCBwb2xpY2llcyB0byBwcm90ZWN0IHdhdGVyIHF1YWxpdHkgYW5kIFNTRnMgaW4gYSB2YXJpZXR5IG9mIGNvYXN0YWwgbGFnb29uIHNldHRpbmdzLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyMi4yODUwMzYwODcwMzYxMzMsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40OTMwNTM0MzYyNzkzLCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyMy43NzM1MTk1MTU5OTEyMSwibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ4MzM2NDEwNTIyNDYxLCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyNS4yNTU1ODQ3MTY3OTY4NzUsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40ODk1NTkxNzM1ODM5ODQsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjI2Ljc0NDA2ODE0NTc1MTk1MywibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjUwNjgwNTQxOTkyMTg3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjguMjI2MTMzMzQ2NTU3NjE3LCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDkwNDYzMjU2ODM1OTQsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjI5LjcxNDYxNjc3NTUxMjY5NSwibGVmdCI6OC4zMzE1OTI1NTk4MTQ0NTMsIndpZHRoIjo0MC40OTQwOTg2NjMzMzAwOCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MzEuMTk2NjgzODgzNjY2OTkyLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDk0MzkyMzk1MDE5NTMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjMyLjY4NTE2NTQwNTI3MzQ0LCJsZWZ0Ijo4LjMzMTU5NjM3NDUxMTcxOSwid2lkdGgiOjQwLjQ4NzQ0NTgzMTI5ODgzLCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozNC4xNjcyMzI1MTM0Mjc3MzQsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDk3NzQ1NTEzOTE2MDE2LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozNS42NTU3MTU5NDIzODI4MSwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40OTg2MzgxNTMwNzYxNywiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MzcuMTM3NzgzMDUwNTM3MTEsImxlZnQiOjEwLjY4NTE4NjM4NjEwODM5OCwid2lkdGgiOjM4LjE0MTEwNTY1MTg1NTQ3LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozOC42MjYyNjY0Nzk0OTIxOSwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40ODcxNTIwOTk2MDkzNzUsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQwLjEwODMzMzU4NzY0NjQ4NCwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40ODk4NTY3MTk5NzA3LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo0MS41OTY4MTMyMDE5MDQzLCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4ODY2NjUzNDQyMzgzLCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo0My4wNzg4ODAzMTAwNTg1OTQsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDgxODYxMTE0NTAxOTUsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQ0LjU2NzM2MzczOTAxMzY3LCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4MzA1ODkyOTQ0MzM2LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo0Ni4wNDk0MzA4NDcxNjc5NywibGVmdCI6OC4zMzMxMDg5MDE5Nzc1MzksIndpZHRoIjo0MC41MDA3NDc2ODA2NjQwNiwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDcuNTM3OTE0Mjc2MTIzMDUsImxlZnQiOjguMzMzMTA4OTAxOTc3NTM5LCJ3aWR0aCI6NDAuNDk1NjA1NDY4NzUsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQ5LjAxOTk4MTM4NDI3NzM0NCwibGVmdCI6OC4zMzMxMDYwNDA5NTQ1OSwid2lkdGgiOjQwLjUwNjgxNjg2NDAxMzY3LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo1MC41MDg0NjQ4MTMyMzI0MiwibGVmdCI6OC4zMzMxMDYwNDA5NTQ1OSwid2lkdGgiOjQwLjQ4ODUwNjMxNzEzODY3LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo1MS45OTA1MjgxMDY2ODk0NSwibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ5NzcyNjQ0MDQyOTY5LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo1My40NzkwMTE1MzU2NDQ1MywibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjMzLjE1OTQ0NjcxNjMwODU5NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NTQuOTYxMDc4NjQzNzk4ODMsImxlZnQiOjEwLjY4MzY3MDA0Mzk0NTMxMiwid2lkdGgiOjM4LjEzNjU5NjY3OTY4NzUsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjU2LjQ0OTU2MjA3Mjc1MzkwNiwibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ4NzE0NDQ3MDIxNDg0NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIxMyJ9fV0%3D.

Warmer waters foster vertical stratification in shallow tropical lakes, curtailing vertical mixing and trapping nutrients in hypoxic bottom layers, which fuels internal loading. Elevated temperatures accelerate microbial decomposition of organic matter, releasing sedimentary phosphorus under anoxic conditions a feedback loop where decaying blooms deplete dissolved oxygen below 2 mg/L, solubilizing iron-bound P and recycling it to surface waters (Haxthausen, 2021; Swann et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1pY2FoIE0uIiwiZmFtaWx5IjoiU3dhbm4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQWxpY2lhIiwiZmFtaWx5IjoiQ29ydMOpcyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBbGV4YW5kZXIgTC4iLCJmYW1pbHkiOiJGb3JyZXN0In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5pY2hvbGFzIFQuIiwiZmFtaWx5IjoiRnJhbXN0ZWQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU3RldmVuIiwiZmFtaWx5IjoiU2Fkcm8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiR2VvZmZyZXkiLCJmYW1pbHkiOiJTY2hsYWRvdyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBbmdlbGEgRGUiLCJmYW1pbHkiOiJQYWxtYeKAkERvdyJ9fV0sImlkcyI6eyJpc3NuIjpbIjEwMTUtMTYyMSIsIjE0MjAtOTA1NSJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMTAwNy9zMDAwMjctMDI0LTAxMDQ1LTIiLCJvcGVuYWxleCI6Ilc0MzkyNzEzMzk5IiwiZXhhIjoiaHR0cHM6Ly9saW5rLnNwcmluZ2VyLmNvbS9hcnRpY2xlLzEwLjEwMDcvczAwMDI3LTAyNC0wMTA0NS0yP2Vycm9yPWNvb2tpZXNfbm90X3N1cHBvcnRlZCZjb2RlPTgzY2Y4ZDg4LWM5YTAtNGNkZC04MjU2LTFhODVlNmFiZGFjMyJ9LCJ0aXRsZSI6IkludGVybmFsIHBob3NwaG9ydXMgbG9hZGluZyBhbHRlcnMgbnV0cmllbnQgbGltaXRhdGlvbiBhbmQgY29udHJpYnV0ZXMgdG8gY3lhbm9iYWN0ZXJpYWwgYmxvb21zIGluIGEgcG9seW1pY3RpYyBsYWtlIiwiYWJzdHJhY3QiOiJBYnN0cmFjdCBDbGVhciBMYWtlLCBhIG1lZGl1bS1zaXplZCBoeXBlcmV1dHJvcGhpYywgcG9seW1pY3RpYyBsYWtlIGluIG5vcnRoZXJuIENhbGlmb3JuaWEsIGhhcyBoYWQgcmVjdXJyaW5nIGhhcm1mdWwgY3lhbm9iYWN0ZXJpYSBibG9vbXMgKEhDQnMpIGZvciBvdmVyIGEgY2VudHVyeSBkZXNwaXRlIHJlZHVjdGlvbnMgaW4gZXh0ZXJuYWwgcGhvc3Bob3J1cyAoUCkgbG9hZGluZ3MuIEludGVybmFsIFAgbG9hZGluZyBjYW4gYWx0ZXIgbnV0cmllbnQgYXZhaWxhYmlsaXR5IGFuZCBsaW1pdGF0aW9uIHN1cHBvcnRpbmcgSENCcyBidXQgaXMgcmFyZWx5IHF1YW50aWZpZWQgb3IgY29tcGFyZWQgd2l0aCBleHRlcm5hbCBsb2Fkcy4gV2UgaGF2ZSBxdWFudGlmaWVkIGV4dGVybmFsIFAgbG9hZHMgZnJvbSAyMDE5IHRvIDIwMjEgZm9yIHRoZSB0aHJlZSBtYWluIHRyaWJ1dGFyaWVzIChhY2NvdW50aW5nIGZvciA0NiUgb2YgdGhlIGZsb3cpIGFuZCBpbnRlcm5hbCBsb2FkaW5ncyB1c2luZyB0d28gbWV0aG9kczogYSBQIG1hc3MgYmFsYW5jZSBhbmQgbW9kZWxlZCByZWxlYXNlIHJhdGVzIG9mIHNvbHVibGUgcmVhY3RpdmUgcGhvc3Bob3J1cyBmcm9tIG94aWMgYW5kIGFub3hpYyBzZWRpbWVudHMuIEluIGFkZGl0aW9uLCB3ZSBjb21iaW5lZCBoaWdoLWZyZXF1ZW5jeSBpbiBzaXR1IG1lYXN1cmVtZW50cyBvZiB3YXRlciB0ZW1wZXJhdHVyZSBhbmQgZGlzc29sdmVkIG94eWdlbiwgZGlzY3JldGUgZ3JhYiBzYW1wbGluZyBmb3IgbnV0cmllbnQgY2hlbWlzdHJ5LCBhbmQgcmVtb3RlIHNlbnNpbmcgdG8gZXhwbG9yZSB0aGUgcG90ZW50aWFsIGRyaXZlcnMgb2YgdGhlIG9ic2VydmVkIHZhcmlhYmlsaXR5IGFuZCBwcm92aWRlIGEgY29tcHJlaGVuc2l2ZSB2aWV3IG9mIHRoZSBzcGF0aW90ZW1wb3JhbCBkeW5hbWljcyBvZiBIQ0JzLiBCeSB1bmRlcnN0YW5kaW5nIHRoZSByZWxhdGl2ZSBjb250cmlidXRpb24gb2YgZXh0ZXJuYWwgYW5kIGludGVybmFsIG51dHJpZW50IGxvYWRpbmdzIGFuZCB0aGUgcmVsYXRpb25zaGlwIGJldHdlZW4gZW52aXJvbm1lbnRhbCBwYXJhbWV0ZXJzIGFuZCBIQ0JzLCBpbnRlcmFubnVhbCBibG9vbSB2YXJpYWJpbGl0eSBjYW4gYmUgYmV0dGVyIHByZWRpY3RlZC4gQ29tcGFyYXRpdmUgZXN0aW1hdGVzIG9mIGV4dGVybmFsIGFuZCBpbnRlcm5hbCBwaG9zcGhvcnVzIGxvYWRpbmcgaW5kaWNhdGUgdGhhdCBpbnRlcm5hbCBzb3VyY2VzIGFjY291bnRlZCBmb3IgNzDigJM5NSUgb2YgdGhlIHRvdGFsIFAgaW5wdXQgaW50byB0aGUgc3lzdGVtIGR1cmluZyB0aGUgc3R1ZHkgcGVyaW9kLiBDb250cmFyeSB0byBvdGhlciBsYWtlcywgdGhlIGludGVuc2l0eSBvZiB0aGUgc3VtbWVyIGJsb29tIHNlYXNvbiB3YXMgY29ycmVsYXRlZCB0byB0aGUgdGltaW5nIGFuZCBkdXJhdGlvbiBvZiBhbm94aWEgcmF0aGVyIHRoYW4gdGhlIG1hZ25pdHVkZSBvZiBzcHJpbmcgcnVub2ZmLiBJbnRlcm5hbGx5IHJlbGVhc2VkIFAgc2hpZnRlZCB0aGUgc3lzdGVtIGZyb20gcGhvc3Bob3J1cyB0byBuaXRyb2dlbiBsaW1pdGF0aW9uIGR1cmluZyB0aGUgc3VtbWVyLCBwb3RlbnRpYWxseSBmYXZvcmluZyB0aGUgcHJvbGlmZXJhdGlvbiBvZiBuaXRyb2dlbi1maXhpbmcgY3lhbm9iYWN0ZXJpYS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAwNy9zMDAwMjctMDI0LTAxMDQ1LTIiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vbGluay5zcHJpbmdlci5jb20vY29udGVudC9wZGYvMTAuMTAwNy9zMDAwMjctMDI0LTAxMDQ1LTIucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjQtMDMtMTIiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTExIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJBcXVhdGljIFNjaWVuY2VzIiwiaXNzdWUiOjIsInZvbHVtZSI6ODYsImltcGFjdEZhY3RvciI6IjIuODQifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiU3ByaW5nZXIgU2NpZW5jZStCdXNpbmVzcyBNZWRpYSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NzUsImNpdGVkQnlDb3VudCI6MjF9fSwicXVvdGUiOiJDb250cmFyeSB0byBvdGhlciBsYWtlcywgdGhlIGludGVuc2l0eSBvZiB0aGUgc3VtbWVyIGJsb29tIHNlYXNvbiB3YXMgY29ycmVsYXRlZCB0byB0aGUgdGltaW5nIGFuZCBkdXJhdGlvbiBvZiBhbm94aWEgcmF0aGVyIHRoYW4gdGhlIG1hZ25pdHVkZSBvZiBzcHJpbmcgcnVub2ZmLiBJbnRlcm5hbGx5IHJlbGVhc2VkIFAgc2hpZnRlZCB0aGUgc3lzdGVtIGZyb20gcGhvc3Bob3J1cyB0byBuaXRyb2dlbiBsaW1pdGF0aW9uIGR1cmluZyB0aGUgc3VtbWVyLCBwb3RlbnRpYWxseSBmYXZvcmluZyB0aGUgcHJvbGlmZXJhdGlvbiBvZiBuaXRyb2dlbi1maXhpbmcgY3lhbm9iYWN0ZXJpYS4gLSBbQ2FsY2l1bSBhbmQgcGhvc3BoYXRlIG1ldGFib2xpYyBkaXNvcmRlcnNdKGh0dHBzOi8vbGluay5zcHJpbmdlci5jb20vc3ViamVjdHMvY2FsY2l1bS1hbmQtcGhvc3BoYXRlLW1ldGFib2xpYy1kaXNvcmRlcnMpIEV1dHJvcGhpY2F0aW9uIHBvc2VzIGEgZ3JhdmUgdGhyZWF0IHRvIGxlbnRpYyBlY29zeXN0ZW1zIGdsb2JhbGx5LCBwcm9tb3RpbmcgdGhlIHByb2xpZmVyYXRpb24gb2YgaGFybWZ1bCBjeWFub2JhY3RlcmlhIGJsb29tcyAoSENCcykgdGhhdCByZWR1Y2UgYXF1YXRpYyBiaW9kaXZlcnNpdHksIHlpZWxkaW5nIGluY3JlYXNlZCB3YXRlciB0cmVhdG1lbnQgY29zdHMsIGFuZCB0aHJlYXRlbmluZyBodW1hbiBoZWFsdGggdGhyb3VnaCBibG9vbS1wcm9kdWNlZCBjeWFub3RveGlucyAoQXLDoW96IGV0IGFsLiJ9LHsib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkthcmwgUmFzbXVzIEF1Z3VzdCIsImZhbWlseSI6IkhheHRoYXVzZW4ifX1dLCJpZHMiOnsiaXNzbiI6W10sImlzYm4iOltdLCJtYWciOiIzMTc3MTEzMjA3Iiwib3BlbmFsZXgiOiJXMzE3NzExMzIwNyJ9LCJ0aXRsZSI6IlNlZGltZW50IG1pY3JvYmlhbCBmdWVsIGNlbGxzIGZvciBsYWtlIHJlc3RvcmF0aW9uIiwiYWJzdHJhY3QiOiJFdXRyb3BoaWNhdGlvbiBvZiBsYWtlcyBpcyB1YmlxdWl0b3VzLCBhcyAzMC00MCAlIG9mIHRoZSB3b3JsZOKAmXMgbGFrZXMgYXJlIGV1dHJvcGhpYywgYW5kIGNvbmRpdGlvbnMgY29udGludW91c2x5IGRldGVyaW9yYXRpbmcuIEV1dHJvcGhpY2F0aW9uIGlzIGEgY29uZGl0aW9uIG9mIG92ZXJmZXJ0aWxpemF0aW9uLCB3aGVyZSBhbGwgcmVxdWlyZW1lbnRzIGZvciByYXBpZCBncm93dGggb2YgYWxnYWUgYXJlIG1ldCwgcmVzdWx0aW5nIGluIGFsZ2FsIGJsb29tcy4gVGhpcyBjYXVzZXMgYSBzZWxmLXN1c3RhaW5lZCwgdmljaW91cyBjeWNsZSB3aGVyZSBwb29yIGVjb2xvZ2ljYWwgY29uZGl0aW9ucyBhcmUgbWFpbnRhaW5lZCB1bnRpbCBudXRyaWVudHMgYXJlIGxvc3QgZnJvbSB0aGUgbGFrZSBvdmVyIG1hbnkgeWVhcnMuIExha2UgcmVzdG9yYXRpb24gc2Vla3MgdG8gYnJlYWsgdGhpcyB2aWNpb3VzIGN5Y2xlLiBJbmFkZXF1YXRlIGF2YWlsYWJpbGl0eSBvZiBhbnkgc2luZ2xlIG51dHJpZW50IGluIHdpbGwgbGltaXQgYWxnYWwgZ3Jvd3RoLCB0eXBpY2FsbHkgcGhvc3Bob3J1cyBpcyB0YXJnZXRlZC4gVGhlIHNjb3BlIG9mIHRoaXMgUGhEIHRoZXNpcyB3YXMgdG8gZGV2ZWxvcCwgdGVzdCBhbmQgb3B0aW1pemUgc2VkaW1lbnQgbWljcm9iaWFsIGZ1ZWwgY2VsbHMgKFNNRkMpIGFzIGEgbm92ZWwgbWV0aG9kIGZvciBsYWtlIHJlc3RvcmF0aW9uLiBUaGUgdGhlc2lzIHNwYW5zIHRoZSB0d28gbWFpbiB0b3BpY3Mgb2YgU01GQ3MgYW5kIGxha2UgcmVzdG9yYXRpb24gdGVjaG5pcXVlcy4gTGFrZSByZXN0b3JhdGlvbiB0ZWNobmlxdWVzIGFyZSByZXZpZXdlZCwgcHJvdmlkaW5nIHVuZGVyc3RhbmRpbmcgb2YgdGhlIHVuZGVybHlpbmcgbWVjaGFuaXNtcy4gU01GQ3MgYXJlIGFwcHJvYWNoZWQgZnJvbSBhbiBlbmdpbmVlcmluZyBwZXJzcGVjdGl2ZSBhbmQgYSBmb2N1cyBvbiBsYWtlIHJlbWVkaWF0aW9uLiBUaGUgc3BlY2lmaWMgb2JqZWN0aXZlcyB3ZXJlIHRvIHRlc3QgYSBsb3ctY29zdCBlbGVjdHJvZGUgbWF0ZXJpYWwgZm9yIGl0cyBkdXJhYmlsaXR5LCBwcmFjdGljYWxpdHksIGFuZCBwZXJmb3JtYW5jZS4gUGVyZm9ybWFuY2Ugd2FzIGFzc2Vzc2VkIGFzIHRoZSBhYmlsaXR5IHRvIGxpbWl0IHBob3NwaG9ydXMgcmVsZWFzZSBmcm9tIHNlZGltZW50cywgaW5oaWJpdCBtZXRoYW5lIHJlbGVhc2UsIGFuZCB0aGUgYWRkZWQgYmVuZWZpdHMgb2YgZGVncmFkaW5nIHBvbGx1dGFudHMsIGFuZCBnZW5lcmF0aW5nIGVsZWN0cmljaXR5LiBQcm9vZiBvZiBjb25jZXB0IHdhcyBzaG93biBpbiBsYWJvcmF0b3J5IHJlYWN0b3JzICgxIEwpLCB0aGUgZWZmZWN0IG9mIHVwc2NhbGluZyB0byAyMDAgTCBpbiBuYXR1cmFsIGVudmlyb25tZW50IHdhcyBzdHVkaWVkIGluIG91dGRvb3IgbWVzb2Nvc21zLCBhbmQgdGhlbiB0ZXN0ZWQgYXQgcGlsb3Qgc2NhbGUgaW4gYSBldXRyb3BoaWMgZnJlc2h3YXRlciBsYWtlLiBQaWxvdCBpbnN0YWxsYXRpb25zIHdlcmUgdGhlIGxhcmdlc3QgaW4gdGhlIHdvcmxkIG9mIHRoZWlyIGtpbmQgYW5kIHdlcmUgb3BlcmF0ZWQgZm9yIDIgeWVhcnMuIFN0YWlubGVzcyBzdGVlbCB3YXMgc2VsZWN0ZWQgZm9yIHRlc3RpbmcgYXMgZWxlY3Ryb2RlIG1hdGVyaWFsIGR1ZSB0byBpdHMgbG93IGNvc3QsIGR1cmFiaWxpdHksIGFuZCBhdmFpbGFiaWxpdHkuIFBhaXJzIG9mIGVsZWN0cm9kZXMgd2VyZSBlbWJlZGRlZCBpbiB0aGUgc2VkaW1lbnQgKHRoaXMgaXMgY2FsbGVkIHRoZSBhbm9kZSkgYW5kIHN1c3BlbmRlZCBpbiB0aGUgb3Zlcmx5aW5nIHdhdGVyICh0aGlzIGlzIGNhbGxlZCB0aGUgY2F0aG9kZSkuIFRoZSB0d28gZWxlY3Ryb2RlcyB3ZXJlIGNvbm5lY3RlZCBhY3Jvc3MgYW4gZXh0ZXJuYWwgcmVzaXN0b3Igd2l0aCBlbGVjdHJpY2FsIHdpcmVzLiBNaWNyb2JlcyBpbiB0aGUgc2VkaW1lbnQgdXNlIHRoZSBhbm9kZSBwYWlyIGFzIGVsZWN0cm9uIGFjY2VwdG9yLCBlbmFibGluZyByZXNwaXJhdGlvbiBpbiB0aGUgYW5veGljIHNlZGltZW50LiBFbGVjdHJvbnMgdHJhdmVsIGZyb20gdGhlIHNlZGltZW50IHZpYSB0aGUgZWxlY3RyaWNhbCBjb25uZWN0b3IgdG8gdGhlIG92ZXJseWluZyBjYXRob2RlIHdoZXJlIG94eWdlbiBpbiB0aGUgd2F0ZXIgaXMgdGhlIHRlcm1pbmFsIGVsZWN0cm9uIGFjY2VwdG9yIGNvbXBsZXRpbmcgdGhlIGNpcmN1aXQuIEFzIGFuIGFkZGVkIGJlbmVmaXQsIGRlZ3JhZGF0aW9uIG9mIHBvbGx1dGFudHMgdXNpbmcgU01GQyB3YXMgdGVzdGVkIGluIG1hcmluZSBoYXJib3VyIHNlZGltZW50cywgYnV0IG5vIGNvbmNsdXNpdmUgcmVzdWx0cyBvYnRhaW5lZC4gTGFib3JhdG9yeSByZWFjdG9ycyB3ZXJlIG9wZXJhdGVkIGZvciBtb3JlIHRoYW4gMjAwIGRheXMgdXNpbmcgcGV0cm9sZXVtIGFzIHNvbGUgY2FyYm9uIHNvdXJjZSwgY29uZmlybWluZyBvaWwgZGVncmFkYXRpb24gYnkgU01GQ3MuIEEgcmVtb3ZhbCByYXRlIG9mIDAuOSBnIHBldHJvbGV1bSBtLTMgZC0xIHJlYWN0b3Igdm9sdW1lIHdhcyBlc3RpbWF0ZWQgZnJvbSBtZWFzdXJlZCBlbGVjdHJpY2FsIG91dHB1dC4gVGhpcyB3YXMgZG9uZSBieSBhc3N1bXB0aW9uIG9mIHRoZSBjaGVtaWNhbCByZWFjdGlvbiBvY2N1cnJpbmcsIGZyb20gdGhpcyB0aGUgbnVtYmVyIG9mIGVsZWN0cm9ucyB0cmFuc2ZlcnJlZCBmcm9tIG9pbCB0byB3YXRlciB3YXMga25vd24gYW5kIGNvdWxkIGJlIGNvcnJlbGF0ZWQgdG8gdGhlIG1lYXN1cmVkIGZsb3cgb2YgZWxlY3Ryb25zIChjdXJyZW50KSB0aHJvdWdoIHRoZSBleHRlcm5hbCBjaXJjdWl0LiBTdGFpbmxlc3Mgc3RlZWwgZWxlY3Ryb2RlcyB3ZXJlIG9wZXJhdGVkIGluIGZyZXNod2F0ZXIgbGFrZSBlbnZpcm9ubWVudHMgZm9yIG1vcmUgdGhhbiA0IHllYXJzIHdpdGggbWluaW1hbCBkYW1hZ2UgdG8gdGhlIG1hdGVyaWFsLiBUaGUgZWxlY3Ryb2RlIGlzIHRoZSBtYWluIG1hdGVyaWFsIGNvc3QgaW4gc3VjaCBzeXN0ZW1zLiBIb3dldmVyLCBBSVNJMzE2IHN0YWlubGVzcyBzdGVlbCBzaW1pbGFyIHRvIHdoYXQgd2FzIHVzZWQgaW4gdGhpcyB0aGVzaXMgd2FzIGF0IHRoZSB0aW1lIG9mIHdyaXRpbmcgYXZhaWxhYmxlIGF0IGJ1bGsgY29zdCBvZiAwLjA5MSDigqwgbS0yLCBjb3JyZXNwb25kaW5nIHRvIDkxMCDigqwgYWNyZS0xLCBzaWduaWZpY2FudGx5IGxvd2VyIHRoYW4gb3RoZXIgY29tbW9uIG1hdGVyaWFscy4gQ2F0aG9kZXMgYXQgbGFyZ2Ugc2NhbGUgY291bGQgYmUgbW92ZWQgaG9yaXpvbnRhbGx5IGFuZCB2ZXJ0aWNhbGx5IHdpdGggbWlub3IgaW1wYWN0IHRvIHBlcmZvcm1hbmNlLiBQaWxvdCBzY2FsZSBTTUZDcyBnZW5lcmF0ZWQgMC4zODMgbVcgbS0yIHN1c3RhaW5lZCBhbmQgMjcuNCBtVyBtLTIgcGVhayBvdXRwdXQuIFRoZSBzeXN0ZW0gd2FzIGxpbWl0ZWQgYnkgdGhlIGNhdGhvZGUgc3VyZmFjZSBhcmVhLCBtZWFuaW5nIG11bHRpLWxheWVyIGNhdGhvZGVzIHNob3VsZCBiZSB0ZXN0ZWQgaW4gdGhlIGZ1dHVyZSBpZiBlbGVjdHJpY2FsIHBvd2VyIHByb2R1Y3Rpb24gaXMgdGhlIGdvYWwuIFRoZSBjYXRob2RlIHNob3VsZCBiZSBwbGFjZWQgYXQgYSBkZXB0aCB0aGF0IGVuc3VyZXMgaGlnaCBkaXNzb2x2ZWQgb3h5Z2VuIGNvbmNlbnRyYXRpb25zIHRvIGF2b2lkIHRoZSBzeXN0ZW0gdG8gYmUgbGltaXRlZC4gSG93ZXZlciwgdG9vIGhpZ2ggZWxlY3RyaWNpdHkgZ2VuZXJhdGlvbiB3YXMgZm91bmQgY291bnRlcnByb2R1Y3RpdmUgZm9yIHJlbWVkaWF0aW9uIHB1cnBvc2VzIGJlY2F1c2UgdGhpcyB3b3VsZCBpbmNyZWFzZSBzZWRpbWVudCB0dXJub3ZlciByYXRlLCByZWxlYXNpbmcgcG9sbHV0YW50cyBzdG9yZWQgaW4gdGhlIHNlZGltZW50cy4gUCBiaW5kaW5nIHdhcyBub3QgZm91bmQgdG8gcmVxdWlyZSBoaWdoIHR1cm5vdmVyIG9mIHNlZGltZW50IG9yZ2FuaWMgbWF0dGVyLiBUaGUgdHVybm92ZXIgY2FuIGJlIGNvbnRyb2xsZWQgYnkgY2hhbmdpbmcgdGhlIGV4dGVybmFsIHJlc2lzdGFuY2UuIEhpZ2ggcmVzaXN0YW5jZSBsaW1pdHMgdGhlIGVsZWN0cmljYWwgcGVyZm9ybWFuY2UsIGxvdyByZXNpc3RhbmNlIGluY3JlYXNlcyBzZWRpbWVudCB0dXJub3ZlciBhbmQgcmlza3MgZGVzdGFiaWxpemluZyB0aGUgc3lzdGVtLiBUbyBhY2hpZXZlIHJlZHVjZWQgY29uY2VudHJhdGlvbnMgb2YgcGhvc3BoYXRlIGluIHRoZSB3YXRlciBjb2x1bW4sIGFub2RlcyBtdXN0IGJlIHBsYWNlZCBpbiB0aGUgc2VkaW1lbnQgc3VyZmFjZSBzaW5jZSBhIGZldyBtaWxsaW1ldHJlcyBvZiBzZWRpbWVudCBtYXkgY29udGFpbiBhbmQgcmVsZWFzZSBlbm91Z2ggUCB0byBzdXN0YWluIGV1dHJvcGhpY2F0aW9uLiBQaG9zcGhhdGUgd2FzIG1lYXN1cmVkIGluIHRoZSB3YXRlciBjb2x1bW4gbmVhciB0aGUgc2VkaW1lbnQsIGluIHN5c3RlbXMgd2l0aCBhbmQgd2l0aG91dCBlbGVjdHJvZGVzLiBQaG9zcGhhdGUgY29uY2VudHJhdGlvbnMgd2VyZSBzdWNjZXNzZnVsbHkgcmVkdWNlZCBieSB1cCB0byA5NSAlIGluIDItY2hhbWJlciBsYWJvcmF0b3J5IHJlYWN0b3JzLCBhbmQgYnkgdXAgdG8gOTggJSBpbiB0aGUgcGlsb3QgaW5zdGFsbGF0aW9uLiBJbiB0aGUgcGlsb3QgZXhwZXJpbWVudCwgcGhvc3BoYXRlIGNvbmNlbnRyYXRpb25zIGFib3ZlIGVsZWN0cm9kZXMgd2VyZSBsb3dlciBpbiBhbGwgbWVhc3VyZW1lbnRzLCBjb21wYXJlZCB0byBjb250cm9scy4gU2VkaW1lbnQgcmVkb3ggcG90ZW50aWFscyBpbmNyZWFzZWQgZnJvbSAtNDAwIG1WIHZzIHN0YW5kYXJkIGh5ZHJvZ2VuIGVsZWN0cm9kZSAoU0hFKSB0byAtMTc1IG1WIHZzIFNIRSwgY29ycmVzcG9uZGluZyB0byBhIG1vdmUgZnJvbSBhIG1ldGhhbm9nZW5ldGljIGRvbWFpbiB0byBzdWxwaHVyIHJlZHVjaW5nLiBBbmFseXNpcyBvZiB0aGUgbWljcm9iaWFsIGNvbW11bml0eSBpbmRpY2F0ZWQgZW5oYW5jZWQgc3VscGh1ciBjeWNsaW5nIHdoZXJlIGVsZWN0cm9kZXMgb3hpZGl6ZSBzdWxwaGlkZSB3aGljaCB0aGUgYmFjdGVyaWEgYWdhaW4gcmVkdWNlIHRvIG9idGFpbiBlbmVyZ3kuIEl0IHdhcyBoeXBvdGhlc2lzZWQgdGhhdCBTTUZDIHByb21vdGUgaGlnaGVyIHJlZG94IHBvdGVudGlhbHMgYnkgZmFjaWxpdGF0aW5nIG94aWRhdGlvbiBvZiBzdWxwaHVyIGFuZCBpcm9uIHNwZWNpZXMsIGluaGliaXRpbmcgSDJTIGZvcm1hdGlvbiBhbmQgRmUgc2NhdmVuZ2luZyBieSBGZVMgZm9ybWF0aW9uLCB0aHVzIGFsbG93aW5nIGZvciBQIGJpbmRpbmcuIEEgbG93ZXIgcmVsYXRpdmUgYWJ1bmRhbmNlIG9mIG1ldGhhbm9nZW5zIG9uIGVsZWN0cm9kZXMgY29tcGFyZWQgdG8gY29udHJvbHMgc3VnZ2VzdHMgc3VwcHJlc3Npb24gb3Igb3V0Y29tcGV0aW5nIG9mIG1ldGhhbm9nZW5zLlF1YW50aWZpY2F0aW9uIG9mIG1ldGhhbmUgcHJvZHVjdGlvbiB3YXMgZG9uZSBvbiB0aGUgYmFzaXMgb2YgYSBDSDQ6Q08yIHJhdGlvIHRvIGFsbG93IGNvbXBhcmFiaWxpdHkgYmV0d2VlbiBzeXN0ZW1zLiBNZXRoYW5lIGVtaXNzaW9uIHdhcyByZWR1Y2VkIGJ5IHVwIHRvIDk5ICUgYW5kIG1haW50YWluZWQgaW5oaWJpdGlvbiBmb3Igc2V2ZXJhbCBtb250aHMgaW4gbGFib3JhdG9yeSBzeXN0ZW1zIHdpdGggZWxlY3Ryb2Rlcy4gSW4gdGhlIHBpbG90IGluc3RhbGxhdGlvbnMgbm8gZ2FzIHNhbXBsZXMgY291bGQgYmUgY29sbGVjdGVkLCBhbmQgbm8gQ08yOkNINCByYXRpb3Mgd2VyZSBvYnRhaW5lZC4gQWVyYXRpb24gaXMgdGhlIGN1cnJlbnQgcmVzdG9yYXRpb24gdGVjaG5pcXVlIHRoYXQgbW9zdCBjbG9zZWx5IHJlc2VtYmxlcyB0aGUgcHJvcG9zZWQgU01GQyB0ZWNobmlxdWUuIEFlcmF0aW9uIGhhcyBwcmV2aW91c2x5IGZhaWxlZCB0byByZWR1Y2UgUCByZWxlYXNlIGluIHNldmVyYWwgc2VwYXJhdGUgaW52ZXN0aWdhdGlvbnMuIFNNRkNzIGFzIGEgcmVtZWRpYXRpb24gdG9vbCB3YXMgZm91bmQgdG8gb2ZmZXIgc2V2ZXJhbCBiZW5lZml0cyBvdmVyIGNvbnZlbnRpb25hbCBtZXRob2RzOiBTTUZDcyBvZmZlcnMgY29udGludW91cyBlZmZlY3QsIHdoaWxlIG90aGVyIHRlY2huaXF1ZXMgcmVxdWlyZSBjb250aW51b3VzIG9yIHJlcGVhdGVkIHRyZWF0bWVudC4gU01GQ3MgY2FuIGJlIGNvbnRyb2xsZWQgaW4gcmVhbC10aW1lIGJ5IGFsdGVyaW5nIHRoZSBleHRlcm5hbCByZXNpc3RvciBhbmQgY2FuIGJlIGVhc2lseSBtb25pdG9yZWQgYnkgbWVhc3VyaW5nIHRoZSBwcm9kdWNlZCBlbGVjdHJpY2FsIG91dHB1dC4gSWYgaW1wbGVtZW50ZWQgYXQgbGFyZ2Ugc2NhbGUgaXQgaXMgZXhwZWN0ZWQgdGhhdCB0aGUgZWxlY3Ryb2RlIGZpZWxkcyB3aWxsIGhlbHAgbWFpbnRhaW4gbG93IHBob3NwaGF0ZSBjb25jZW50cmF0aW9ucyBpbiB0aGUgd2F0ZXIgZm9yIGRlY2FkZXMgd2l0aCBtaW5pbWFsIGludGVydmVudGlvbiwgYW5kIGFmdGVyIHNvbWUgdGltZSwgYmVudGhpYyBwbGFudHMgYXJlIGV4cGVjdGVkIHRvIHJldHVybiwgdGh1cyBtYWludGFpbmluZyB0aGUgYmFsYW5jZSBoZW5jZSBmb3J0aC4iLCJ1cmwiOiJodHRwczovL2xvY2FsLmZvcnNrbmluZ3Nwb3J0YWwuZGsvbG9jYWwvZGtpLWNnaS93cy9jcmlzLWxpbms%2Fc3JjPWR0dSZpZD1kdHUtZDY2ZDMzNzAtMjU2Zi00ODg1LWEwOWUtMzM2YTkyNGY0YmVkJnRpPVNlZGltZW50JTIwbWljcm9iaWFsJTIwZnVlbCUyMGNlbGxzJTIwZm9yJTIwbGFrZSUyMHJlc3RvcmF0aW9uIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2JhY2tlbmQub3JiaXQuZHR1LmRrL3dzL2ZpbGVzLzI0ODA2OTU4OS9UaGVzaXNfb25saW5lX3ZlcnNpb25fS2FybF9IYXh0aGF1c2VuLnBkZiIsImlzc3VlZERhdGUiOiIyMDIxLTAxLTAxIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0wOCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiUmVzZWFyY2ggUG9ydGFsIERlbm1hcmsiLCJpbXBhY3RGYWN0b3IiOiIwLjA0In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6NzZ9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiVGVjaG5pY2FsIFVuaXZlcnNpdHkgb2YgRGVubWFyayJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MCwiY2l0ZWRCeUNvdW50IjoxfX0sInF1b3RlIjoiRXh0ZXJuYWwgaW5wdXRzIGluY3JlYXNlIFAgY29uY2VudHJhdGlvbiBpbiAgdGhlIHdhdGVyLCBjYXVzaW5nIGFsZ2FsIGJsb29tcyB0aGF0IHNoYWRvdyB0aGUgbGFrZSBib3R0b20sIGFuZCBiZW50aGljIG1hYy1yb3BoeXRlcyByZWNlZGUuIFByZWRhdG9yeSBmaXNoIGFyZSB1bmFibGUgdG8gaHVudCwgYW5kIHRoZWlyIGZvb2Qgc291cmNlIGlzICBkaXNydXB0ZWQgc3VjaCBhcyBmaXNoIGZlZWRpbmcgb24gYmVudGhpYyBpbnZlcnRlYnJhdGVzIGFuZCBib3R0b20tZHdlbGxpbmcgYW5pbWFscyBhcmUgc3VmZm9jYXRlZCAoVmFuZGVyIFphbmRlbiBhbmQgVmFkZWJvbmNvZXVyIDIwMDIpLCBzbyB0aGF0IHBsYW5rdGl2b3JvdXMgZmlzaCBzcGVjaWVzIGRvbWluYXRlLiBUaGlzIGluY3JlYXNlcyBwcmVkYXRpb24gb24gem9vIHBsYW5rdG9uICBhbmQgcmVkdWNlcyBncmF6aW5nIG9uIGFsZ2FlLiBBbGdhZSBwcm9saWZlcmF0ZSwgZGllLCBhbmQgc2luayB0byB0aGUgYm90dG9tLCAgY29uc3VtaW5nIG94eWdlbiB3aGVuIGRlZ3JhZGVkLCB3aGljaCBsZWFkcyB0byBveHlnZW4gZGVmaWNpdCBpbiB0aGUgYm90LXRvbSwga2lsbGluZyBmaXNoIChXdXJ0c2JhdWdoIGV0IGFsLiAyMDE5KS4gRmlndXJlIDI6IFZpY2lvdXMgY3ljbGVzIG9mIGV1dHJvcGhpY2F0aW9uLiBMZWZ0OiBJbnRlcmxpbmtlZCBwcm9jZXNzZXMgc3VzdGFpbmluZyBldXRyb3BoaS1jYXRpb24gaW4gYSBsYWtlLiBCbHVlOiBwcm9jZXNzZXMgaW4gd2F0ZXIsIGdyZWVuOiBwcm9jZXNzZXMgaW52b2x2aW5nIG1pY3JvYWxnYWUsIGdyZXk6IHNlZC1pbWVudCByZWxhdGVkIHByb2Nlc3NlcywgeWVsbG93OiBwcm9jZXNzZXMgaW52b2x2aW5nIGZhdW5hLiBSaWdodCwgZGFzaGVkIGxpbmU6IFRoZSBvcmRlciBpbiB3aGljaCBlbGVjdHJvbiBhY2NlcHRvcnMgYXJlIGNvbnN1bWVkIGluIHRoZSBhYnNlbmNlIG9mIE8yIHNob3duIGFzIGNvbmNlbnRyYXRpb24gdmVyc3VzICBkZXB0aCBpbiBzZWRpbWVudCwgYW5kIGF0IHdoaWNoIHBvdGVudGlhbCB0aGlzIG9jY3VycyAoQmVybmVyIDE5ODEsIGJ5IGNvdXJ0ZXN5IG9mIHRoZSBzb2NpZXR5ICBmb3Igc2VkaW1lbnRhcnkgZ2VvbG9neSkuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxNywiYmJveCI6eyJ0b3AiOjc5LjkzMzE1MTI0NTExNzE5LCJsZWZ0IjoxNC4yOTU1MDM2MTYzMzMwMDgsIndpZHRoIjo3MS45Mzg5MzQzMjYxNzE4OCwiaGVpZ2h0IjoyLjA1MzQ3NzI4NzI5MjQ4MDV9fSx7InBhZ2VOdW1iZXIiOjE3LCJiYm94Ijp7InRvcCI6ODEuOTcxMzQzOTk0MTQwNjIsImxlZnQiOjE0LjI5NTUwMzYxNjMzMzAwOCwid2lkdGgiOjcxLjMzMzAwMDE4MzEwNTQ3LCJoZWlnaHQiOjIuMDUzNDc3Mjg3MjkyNDgwNX19LHsicGFnZU51bWJlciI6MTcsImJib3giOnsidG9wIjo4NC4wMjM3OTYwODE1NDI5NywibGVmdCI6MTQuMjk1NTAzNjE2MzMzMDA4LCJ3aWR0aCI6NzEuODEyMDE5MzQ4MTQ0NTMsImhlaWdodCI6Mi4wNTM0NzcyODcyOTI0ODA1fX0seyJwYWdlTnVtYmVyIjoxNywiYmJveCI6eyJ0b3AiOjg2LjA2MTk5NjQ1OTk2MDk0LCJsZWZ0IjoxNC4yOTU1MDM2MTYzMzMwMDgsIndpZHRoIjo3MS45NTczNTkzMTM5NjQ4NCwiaGVpZ2h0IjoyLjA1MzQ3NzI4NzI5MjQ4MDV9fSx7InBhZ2VOdW1iZXIiOjE4LCJiYm94Ijp7InRvcCI6OS45MjY0ODc5MjI2Njg0NTcsImxlZnQiOjE0LjI5NTUwMzYxNjMzMzAwOCwid2lkdGgiOjcxLjk3NDA2MDA1ODU5Mzc1LCJoZWlnaHQiOjEwLjIzNDc3NzQ1MDU2MTUyM319LHsicGFnZU51bWJlciI6MTgsImJib3giOnsidG9wIjo0NC43MzQwNDMxMjEzMzc4OSwibGVmdCI6MTQuMjk1NTAzNjE2MzMzMDA4LCJ3aWR0aCI6NzEuODkzNDMyNjE3MTg3NSwiaGVpZ2h0IjoxMC40MDMyODk3OTQ5MjE4NzV9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI5MyJ9fV0%3D. Studies in polymictic Indian lakes reveal that summer anoxia duration correlates directly with bloom intensity, shifting systems from N- to P-limitation and favoring N₂-fixing cyanobacteria that exploit atmospheric nitrogen (Nõges et al., 2020; Swann et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRpaW5hIiwiZmFtaWx5IjoiTsO1Z2VzIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hc2ltZSIsImZhbWlseSI6IkphbmF0aWFuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlJlZXQiLCJmYW1pbHkiOiJMYXVnYXN0ZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQZWV0ZXIiLCJmYW1pbHkiOiJOw7VnZXMifX1dLCJpZHMiOnsiaXNzbiI6WyIyMzUxLTk4OTQiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMTYvai5nZWNjby4yMDIwLmUwMTM2OSIsIm1hZyI6IjMxMDU0NDYwOTAiLCJvcGVuYWxleCI6IlczMTA1NDQ2MDkwIiwiZXhhIjoiaHR0cHM6Ly93d3cuc2NpZW5jZWRpcmVjdC5jb20vc2NpZW5jZS9hcnRpY2xlL3BpaS9TMjM1MTk4OTQyMDMwOTEwMCJ9LCJ0aXRsZSI6IlBvc3Qtc292aWV0IGNoYW5nZXMgaW4gbml0cm9nZW4gYW5kIHBob3NwaG9ydXMgc3RvaWNoaW9tZXRyeSBpbiB0d28gbGFyZ2Ugbm9uLXN0cmF0aWZpZWQgbGFrZXMgYW5kIHRoZSBpbXBhY3Qgb24gcGh5dG9wbGFua3RvbiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2ouZ2VjY28uMjAyMC5lMDEzNjkiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2ouZ2VjY28uMjAyMC5lMDEzNjkiLCJpc3N1ZWREYXRlIjoiMjAyMC0xMS0yMCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTEiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6Ikdsb2JhbCBFY29sb2d5IGFuZCBDb25zZXJ2YXRpb24iLCJ2b2x1bWUiOjI0LCJpbXBhY3RGYWN0b3IiOiIzLjU5In0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkVsc2V2aWVyIEJWIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo3MSwiY2l0ZWRCeUNvdW50IjoxNH19LCJxdW90ZSI6IlJhbmRvbSBGb3Jlc3QgYW5hbHlzaXMgaW5kaWNhdGVkIGFuIGltcG9ydGFudCByb2xlIG9mIGxpZ2h0IGxpbWl0YXRpb24gaW4gYm90aCBsYWtlcy4gQ29uc3RhbnRseSBsb3dlciBsZXZlbHMgb2YgTiBpbiB0aGUgZGVlcGVyIExha2UgUGVpcHNpIGZhdm91cmVkIE4yLWZpeGluZyBzcGVjaWVzLCB3aGljaCwgYXMgYSBwYXJhZG94LCBiZWNhbWUgUC1saW1pdGVkLiBBcyBjbGltYXRlIHdhcm1pbmcgcmVpbmZvcmNlcyBldXRyb3BoaWNhdGlvbiBwaGVub21lbmEgaW4gbGFrZXMgYnkgaW5jcmVhc2luZyBpbnRlcm5hbCBudXRyaWVudCBsb2FkaW5nIGFuZCBmYXZvdXJpbmcgYmxvb20tZm9ybWluZyBjeWFub2JhY3RlcmlhLCBtb3JlIHN0cmluZ2VudCBtZWFzdXJlcyB3b3VsZCBiZSBuZWVkZWQgdG8gZnVydGhlciBsaW1pdCBudXRyaWVudCBsb2FkcyAoZXNwZWNpYWxseSB0aGF0IG9mIFApIHRvIGxha2VzIHRocm91Z2ggaW1wcm92ZWQgd2FzdGV3YXRlciB0cmVhdG1lbnQgYW5kIGluY3JlYXNlZCBlZmZpY2llbmN5IG9mIGZlcnRpbGlzZXIgYXBwbGljYXRpb24uIExvbmctdGVybSBjaGFuZ2VzIGluIG51dHJpZW50IGxvYWRpbmcgYW5kIHN0b2ljaGlvbWV0cnkgaW4gd2F0ZXIgYm9kaWVzIHJlZmxlY3QgY2hhbmdlcyBpbiBhZ3JpY3VsdHVyYWwgcHJhY3RpY2VzIGFuZCB3YXN0ZXdhdGVyIHRyZWF0bWVudC4ifSx7Im9yaWdpbiI6NSwid29yayI6eyJ0eXBlIjo4LCJzdWJ0eXBlIjp7Im9uZW9mS2luZCI6ImFydGljbGVUeXBlIiwiYXJ0aWNsZVR5cGUiOjF9LCJjb250cmlidXRvcnMiOlt7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNaWNhaCBNLiIsImZhbWlseSI6IlN3YW5uIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFsaWNpYSIsImZhbWlseSI6IkNvcnTDqXMifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQWxleGFuZGVyIEwuIiwiZmFtaWx5IjoiRm9ycmVzdCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJOaWNob2xhcyBULiIsImZhbWlseSI6IkZyYW1zdGVkIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlN0ZXZlbiIsImZhbWlseSI6IlNhZHJvIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikdlb2ZmcmV5IiwiZmFtaWx5IjoiU2NobGFkb3cifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW5nZWxhIERlIiwiZmFtaWx5IjoiUGFsbWHigJBEb3cifX1dLCJpZHMiOnsiaXNzbiI6WyIxMDE1LTE2MjEiLCIxNDIwLTkwNTUiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMDcvczAwMDI3LTAyNC0wMTA0NS0yIiwib3BlbmFsZXgiOiJXNDM5MjcxMzM5OSIsImV4YSI6Imh0dHBzOi8vbGluay5zcHJpbmdlci5jb20vYXJ0aWNsZS8xMC4xMDA3L3MwMDAyNy0wMjQtMDEwNDUtMj9lcnJvcj1jb29raWVzX25vdF9zdXBwb3J0ZWQmY29kZT04M2NmOGQ4OC1jOWEwLTRjZGQtODI1Ni0xYTg1ZTZhYmRhYzMifSwidGl0bGUiOiJJbnRlcm5hbCBwaG9zcGhvcnVzIGxvYWRpbmcgYWx0ZXJzIG51dHJpZW50IGxpbWl0YXRpb24gYW5kIGNvbnRyaWJ1dGVzIHRvIGN5YW5vYmFjdGVyaWFsIGJsb29tcyBpbiBhIHBvbHltaWN0aWMgbGFrZSIsImFic3RyYWN0IjoiQWJzdHJhY3QgQ2xlYXIgTGFrZSwgYSBtZWRpdW0tc2l6ZWQgaHlwZXJldXRyb3BoaWMsIHBvbHltaWN0aWMgbGFrZSBpbiBub3J0aGVybiBDYWxpZm9ybmlhLCBoYXMgaGFkIHJlY3VycmluZyBoYXJtZnVsIGN5YW5vYmFjdGVyaWEgYmxvb21zIChIQ0JzKSBmb3Igb3ZlciBhIGNlbnR1cnkgZGVzcGl0ZSByZWR1Y3Rpb25zIGluIGV4dGVybmFsIHBob3NwaG9ydXMgKFApIGxvYWRpbmdzLiBJbnRlcm5hbCBQIGxvYWRpbmcgY2FuIGFsdGVyIG51dHJpZW50IGF2YWlsYWJpbGl0eSBhbmQgbGltaXRhdGlvbiBzdXBwb3J0aW5nIEhDQnMgYnV0IGlzIHJhcmVseSBxdWFudGlmaWVkIG9yIGNvbXBhcmVkIHdpdGggZXh0ZXJuYWwgbG9hZHMuIFdlIGhhdmUgcXVhbnRpZmllZCBleHRlcm5hbCBQIGxvYWRzIGZyb20gMjAxOSB0byAyMDIxIGZvciB0aGUgdGhyZWUgbWFpbiB0cmlidXRhcmllcyAoYWNjb3VudGluZyBmb3IgNDYlIG9mIHRoZSBmbG93KSBhbmQgaW50ZXJuYWwgbG9hZGluZ3MgdXNpbmcgdHdvIG1ldGhvZHM6IGEgUCBtYXNzIGJhbGFuY2UgYW5kIG1vZGVsZWQgcmVsZWFzZSByYXRlcyBvZiBzb2x1YmxlIHJlYWN0aXZlIHBob3NwaG9ydXMgZnJvbSBveGljIGFuZCBhbm94aWMgc2VkaW1lbnRzLiBJbiBhZGRpdGlvbiwgd2UgY29tYmluZWQgaGlnaC1mcmVxdWVuY3kgaW4gc2l0dSBtZWFzdXJlbWVudHMgb2Ygd2F0ZXIgdGVtcGVyYXR1cmUgYW5kIGRpc3NvbHZlZCBveHlnZW4sIGRpc2NyZXRlIGdyYWIgc2FtcGxpbmcgZm9yIG51dHJpZW50IGNoZW1pc3RyeSwgYW5kIHJlbW90ZSBzZW5zaW5nIHRvIGV4cGxvcmUgdGhlIHBvdGVudGlhbCBkcml2ZXJzIG9mIHRoZSBvYnNlcnZlZCB2YXJpYWJpbGl0eSBhbmQgcHJvdmlkZSBhIGNvbXByZWhlbnNpdmUgdmlldyBvZiB0aGUgc3BhdGlvdGVtcG9yYWwgZHluYW1pY3Mgb2YgSENCcy4gQnkgdW5kZXJzdGFuZGluZyB0aGUgcmVsYXRpdmUgY29udHJpYnV0aW9uIG9mIGV4dGVybmFsIGFuZCBpbnRlcm5hbCBudXRyaWVudCBsb2FkaW5ncyBhbmQgdGhlIHJlbGF0aW9uc2hpcCBiZXR3ZWVuIGVudmlyb25tZW50YWwgcGFyYW1ldGVycyBhbmQgSENCcywgaW50ZXJhbm51YWwgYmxvb20gdmFyaWFiaWxpdHkgY2FuIGJlIGJldHRlciBwcmVkaWN0ZWQuIENvbXBhcmF0aXZlIGVzdGltYXRlcyBvZiBleHRlcm5hbCBhbmQgaW50ZXJuYWwgcGhvc3Bob3J1cyBsb2FkaW5nIGluZGljYXRlIHRoYXQgaW50ZXJuYWwgc291cmNlcyBhY2NvdW50ZWQgZm9yIDcw4oCTOTUlIG9mIHRoZSB0b3RhbCBQIGlucHV0IGludG8gdGhlIHN5c3RlbSBkdXJpbmcgdGhlIHN0dWR5IHBlcmlvZC4gQ29udHJhcnkgdG8gb3RoZXIgbGFrZXMsIHRoZSBpbnRlbnNpdHkgb2YgdGhlIHN1bW1lciBibG9vbSBzZWFzb24gd2FzIGNvcnJlbGF0ZWQgdG8gdGhlIHRpbWluZyBhbmQgZHVyYXRpb24gb2YgYW5veGlhIHJhdGhlciB0aGFuIHRoZSBtYWduaXR1ZGUgb2Ygc3ByaW5nIHJ1bm9mZi4gSW50ZXJuYWxseSByZWxlYXNlZCBQIHNoaWZ0ZWQgdGhlIHN5c3RlbSBmcm9tIHBob3NwaG9ydXMgdG8gbml0cm9nZW4gbGltaXRhdGlvbiBkdXJpbmcgdGhlIHN1bW1lciwgcG90ZW50aWFsbHkgZmF2b3JpbmcgdGhlIHByb2xpZmVyYXRpb24gb2Ygbml0cm9nZW4tZml4aW5nIGN5YW5vYmFjdGVyaWEuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjEwMDcvczAwMDI3LTAyNC0wMTA0NS0yIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2xpbmsuc3ByaW5nZXIuY29tL2NvbnRlbnQvcGRmLzEwLjEwMDcvczAwMDI3LTAyNC0wMTA0NS0yLnBkZiIsImlzc3VlZERhdGUiOiIyMDI0LTAzLTEyIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMSIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiQXF1YXRpYyBTY2llbmNlcyIsImlzc3VlIjoyLCJ2b2x1bWUiOjg2LCJpbXBhY3RGYWN0b3IiOiIyLjg0In0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlNwcmluZ2VyIFNjaWVuY2UrQnVzaW5lc3MgTWVkaWEifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjc1LCJjaXRlZEJ5Q291bnQiOjIxfX0sInF1b3RlIjoiQ29udHJhcnkgdG8gb3RoZXIgbGFrZXMsIHRoZSBpbnRlbnNpdHkgb2YgdGhlIHN1bW1lciBibG9vbSBzZWFzb24gd2FzIGNvcnJlbGF0ZWQgdG8gdGhlIHRpbWluZyBhbmQgZHVyYXRpb24gb2YgYW5veGlhIHJhdGhlciB0aGFuIHRoZSBtYWduaXR1ZGUgb2Ygc3ByaW5nIHJ1bm9mZi4gSW50ZXJuYWxseSByZWxlYXNlZCBQIHNoaWZ0ZWQgdGhlIHN5c3RlbSBmcm9tIHBob3NwaG9ydXMgdG8gbml0cm9nZW4gbGltaXRhdGlvbiBkdXJpbmcgdGhlIHN1bW1lciwgcG90ZW50aWFsbHkgZmF2b3JpbmcgdGhlIHByb2xpZmVyYXRpb24gb2Ygbml0cm9nZW4tZml4aW5nIGN5YW5vYmFjdGVyaWEuIC0gW0NhbGNpdW0gYW5kIHBob3NwaGF0ZSBtZXRhYm9saWMgZGlzb3JkZXJzXShodHRwczovL2xpbmsuc3ByaW5nZXIuY29tL3N1YmplY3RzL2NhbGNpdW0tYW5kLXBob3NwaGF0ZS1tZXRhYm9saWMtZGlzb3JkZXJzKSBFdXRyb3BoaWNhdGlvbiBwb3NlcyBhIGdyYXZlIHRocmVhdCB0byBsZW50aWMgZWNvc3lzdGVtcyBnbG9iYWxseSwgcHJvbW90aW5nIHRoZSBwcm9saWZlcmF0aW9uIG9mIGhhcm1mdWwgY3lhbm9iYWN0ZXJpYSBibG9vbXMgKEhDQnMpIHRoYXQgcmVkdWNlIGFxdWF0aWMgYmlvZGl2ZXJzaXR5LCB5aWVsZGluZyBpbmNyZWFzZWQgd2F0ZXIgdHJlYXRtZW50IGNvc3RzLCBhbmQgdGhyZWF0ZW5pbmcgaHVtYW4gaGVhbHRoIHRocm91Z2ggYmxvb20tcHJvZHVjZWQgY3lhbm90b3hpbnMgKEFyw6FveiBldCBhbC4ifV0%3D. This is exacerbated by reduced wind speeds under changing circulation patterns, prolonging calm periods ideal for scum formation. In Chilika Lagoon, warming has intensified post-monsoon blooms, reducing fish yields by 40% amid DO crashes and scum layers blocking 80% of light penetration (Jilbert et al., 2020; Nair & Nayak, 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRvbSIsImZhbWlseSI6IkppbGJlcnQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUmFvdWzigJBNYXJpZSIsImZhbWlseSI6IkNvdXR1cmUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQnJpYW4iLCJmYW1pbHkiOiJIdXNlciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLYWxldmkiLCJmYW1pbHkiOiJTYWxvbmVuIn19XSwiaWRzIjp7Imlzc24iOlsiMDAxOC04MTU4IiwiMTU3My01MTE3Il0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDA3L3MxMDc1MC0wMjAtMDQ0NTcteCIsIm1hZyI6IjMxMDA0NTkwNTAiLCJvcGVuYWxleCI6IlczMTAwNDU5MDUwIn0sInRpdGxlIjoiUHJlZmFjZTogUmVzdG9yYXRpb24gb2YgZXV0cm9waGljIGxha2VzOiBjdXJyZW50IHByYWN0aWNlcyBhbmQgZnV0dXJlIGNoYWxsZW5nZXMiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAwNy9zMTA3NTAtMDIwLTA0NDU3LXgiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vbGluay5zcHJpbmdlci5jb20vY29udGVudC9wZGYvMTAuMTAwNy9zMTA3NTAtMDIwLTA0NDU3LXgucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjAtMTEtMTYiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA3IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJIeWRyb2Jpb2xvZ2lhIiwiaXNzdWUiOjIxLCJ2b2x1bWUiOjg0NywiaW1wYWN0RmFjdG9yIjoiMy4wMCJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjQzNDMsImxhc3RQYWdlIjo0MzU3fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlNwcmluZ2VyIFNjaWVuY2UrQnVzaW5lc3MgTWVkaWEifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjk1LCJjaXRlZEJ5Q291bnQiOjU4fX0sInF1b3RlIjoiU2ltaWxhciBkZWdyYWRhdGlvbiBvZiBmcmVzaHdhdGVyIGVjb3N5c3RlbXMgaXMgb2JzZXJ2ZWQgd29ybGR3aWRlLCB3aXRoIHRoZSB0aW1lbGluZSBvZiBodW1hbiBwcmVzc3VyZSBkaWN0YXRlZCBieSB0cmVuZHMgaW4gdXJiYW5pemF0aW9uIGFuZCBpbnRlbnNpdmUgYWdyaWN1bHR1cmUgaW4gZGlmZmVyZW50IGdlb2dyYXBoaWMgcmVnaW9ucyAoZS5nLiwgVmVubmVtbyBldCBhbC4sIDIwMDkpLiBUaGUgcHJpbmNpcGFsIGRyaXZlciBvZiBlY29zeXN0ZW0gc3RyZXNzIGluIG1vc3QgYXF1YXRpYyBzeXN0ZW1zIGlzIGV1dHJvcGhpY2F0aW9uLCB0aGUgZXhjZXNzIGdyb3d0aCBvZiBhbGdhZSBkcml2ZW4gYnkgYW50aHJvcG9nZW5pYyBpbnB1dHMgb2YgdGhlIG1hY3JvbnV0cmllbnRzIG5pdHJvZ2VuIChOKSBhbmQgcGhvc3Bob3J1cyAoUCkgKFNtaXRoLCAyMDAzOyBCaXJrIGV0IGFsLiwgMjAyMCkuIE51dHJpZW50cyBhcmUgZW1pdHRlZCBmcm9tIGJvdGggcG9pbnQgYW5kIGRpZmZ1c2Ugc291cmNlcyBpbiB0aGUgaHVtYW4gZW52aXJvbm1lbnQgYW5kIGVudGVyIGFxdWF0aWMgc3lzdGVtcyB2aWEgc3VyZmFjZSBydW5vZmYsIGFzIHdlbGwgYXMgZ3JvdW5kLXdhdGVyIGFuZCBhdG1vc3BoZXJpYyBkZXBvc2l0aW9uIChQYWVybCwgMTk5NykuIEV1dHJvcGhpY2F0aW9uIG9mdGVuIGxlYWRzIHRvIHRoZSBkb21pbmFuY2Ugb2YgcHJpbWFyeSBwcm9kdWN0aW9uIGJ5IGN5YW5vYmFjdGVyaWEsIGluY2x1ZGluZyB0b3hpYyBnZW5lcmEgc3VjaCBhcyBEb2xpY2hvc3Blcm11bSwgQXBoYW5pem9tZW5vbiwgYW5kIE1pY3JvY3lzdGlzIChIdWlzbWFuIGV0IGFsLiwgMjAxOCksIGFzIHdlbGwgYXMgb3h5Z2VuIGRlcGxldGlvbiBhbmQgaGFiaXRhdCBkZWdyYWRhdGlvbiBpbiBsYWtlcyAoQXJlbmQgZXQgYWwuLCAyMDExKS4gQmVjYXVzZSBsYWtlcyBwcm92aWRlIGEgcmFuZ2Ugb2YgY3J1Y2lhbCBlY29zeXN0ZW0gc2VydmljZXMgKFdpbHNvbiAmIENhcnBlbnRlciwgMTk5OSksIHRoZXJlIGlzIGEgZ3Jvd2luZyBuZWVkIGZvciBtYW5hZ2VtZW50IGFwcHJvYWNoZXMgdGhhdCByZXN0b3JlIHdhdGVyIHF1YWxpdHkgaW4gZnJlc2h3YXRlciBzeXN0ZW1zLiBXaXRoaW4gdGhlIGZyYW1ld29yayBvZiBVbml0ZWQgTmF0aW9ucyBTdXN0YWluYWJsZSBEZXZlbG9wLW1lbnQgR29hbCA2IChDbGVhbiBXYXRlciBhbmQgU2FuaXRhdGlvbiksIHNldmVyYWwgZGlzdGluY3QgdGFyZ2V0cyByZWxhdGUgdG8gdGhlIGltcHJvdmVtZW50IG9mIHdhdGVyIHF1YWxpdHkgYW5kIHByb3RlY3Rpb24gb2YgYXF1YXRpYyBlY29zeXN0ZW1zIGJ5IDIwMzAgKFVuaXRlZCBOYXRpb25zLCAyMDE1KS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyOC4yOTg4NjQzNjQ2MjQwMjMsImxlZnQiOjguNTQ5MjA1NzgwMDI5Mjk3LCJ3aWR0aCI6MzkuMzcxNDk0MjkzMjEyODksImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MjkuOTkxMTM0NjQzNTU0Njg4LCJsZWZ0Ijo4LjU0OTIwNTc4MDAyOTI5Nywid2lkdGgiOjM5LjM2NTExOTkzNDA4MjAzLCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMxLjY4MzQwNjgyOTgzMzk4NCwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNjk2NjMyMzg1MjUzOSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozMy4zNjc5NzMzMjc2MzY3MiwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNzU2NzkwMTYxMTMyOCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNS4wNjAyNDkzMjg2MTMyOCwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNzA5NjQwNTAyOTI5NywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNi43NTI1MTc3MDAxOTUzMSwibGVmdCI6OC41NDkyMDg2NDEwNTIyNDYsIndpZHRoIjozOS4zNzY5NjA3NTQzOTQ1MywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC40MzcwODgwMTI2OTUzMSwibGVmdCI6OC41NDkyMDg2NDEwNTIyNDYsIndpZHRoIjozOS4zNjY1Njk1MTkwNDI5NywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0MC4xMjkzNTYzODQyNzczNDQsImxlZnQiOjguNTQ5MjA4NjQxMDUyMjQ2LCJ3aWR0aCI6MzkuMzY0Mzc5ODgyODEyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0MS44MjE2MjQ3NTU4NTkzNzUsImxlZnQiOjguNTQ5MjA4NjQxMDUyMjQ2LCJ3aWR0aCI6MzkuMzY1NjczMDY1MTg1NTUsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDMuNTA2MTk1MDY4MzU5Mzc1LCJsZWZ0Ijo4LjU0OTM4OTgzOTE3MjM2Mywid2lkdGgiOjM5LjM3MDM5MTg0NTcwMzEyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0NS4xOTg0NjM0Mzk5NDE0MDYsImxlZnQiOjguNTQ5NTcyOTQ0NjQxMTEzLCJ3aWR0aCI6MzkuMzY1ODQ0NzI2NTYyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ni44OTA3MzU2MjYyMjA3LCJsZWZ0Ijo4LjU0OTU3Mjk0NDY0MTExMywid2lkdGgiOjM5LjM3Mjk1NTMyMjI2NTYyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0OC41NzUzMDIxMjQwMjM0NCwibGVmdCI6OC41NDk1NzI5NDQ2NDExMTMsIndpZHRoIjozOS4zNzI0MjEyNjQ2NDg0NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1MC4yNjc1NzgxMjUsImxlZnQiOjguNTQ5NTc2NzU5MzM4Mzc5LCJ3aWR0aCI6MzkuMzcxMTI0MjY3NTc4MTI1LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjUxLjk1OTg0MjY4MTg4NDc2NiwibGVmdCI6OC41NDk1NzY3NTkzMzgzNzksIndpZHRoIjozOS4zNjc4MzU5OTg1MzUxNTYsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTMuNTc0MTE5NTY3ODcxMDk0LCJsZWZ0Ijo4LjU0OTU3Njc1OTMzODM3OSwid2lkdGgiOjM5LjM3MDc3NzEzMDEyNjk1LCJoZWlnaHQiOjEuMzM2ODk1NTg1MDYwMTE5Nn19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjU1LjI2NjM5MTc1NDE1MDM5LCJsZWZ0Ijo4LjU0OTc2MDgxODQ4MTQ0NSwid2lkdGgiOjM5LjM2NjU4MDk2MzEzNDc2NiwiaGVpZ2h0IjoxLjMzNjg5NTU4NTA2MDExOTZ9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1Ny4wMjg5NTM1NTIyNDYwOTQsImxlZnQiOjguNTQ5OTQ0ODc3NjI0NTEyLCJ3aWR0aCI6MzkuMzYyMDI2MjE0NTk5NjEsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTguNzEzNTIzODY0NzQ2MDk0LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjkuMjk5OTgyMDcwOTIyODUyLCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjYwLjQwNTc5MjIzNjMyODEyNSwibGVmdCI6MTAuNjIyNDU1NTk2OTIzODI4LCJ3aWR0aCI6MzcuMjg3ODY4NDk5NzU1ODYsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NjIuMDk4MDYwNjA3OTEwMTU2LCJsZWZ0Ijo4LjU0OTk0MzkyMzk1MDE5NSwid2lkdGgiOjM5LjM3MzMyNTM0NzkwMDM5LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjYzLjc4MjYzMDkyMDQxMDE1NiwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzMxMjY5ODM2NDI1OCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2NS40NzQ5MDY5MjEzODY3MiwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNjUxMTk5MzQwODIwMywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2Ny4xNjcxNzUyOTI5Njg3NSwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNjk0ODAxMzMwNTY2NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2OC43ODE0NDgzNjQyNTc4MSwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzczMzg0MDk0MjM4MywiaGVpZ2h0IjoxLjMzNjg5MTUzMTk0NDI3NX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjcwLjU0NDAxMzk3NzA1MDc4LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjM5LjM2OTMwODQ3MTY3OTY5LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjcyLjIzNjI4OTk3ODAyNzM0LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjM5LjM3MjAzMjE2NTUyNzM0NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo3My45MjA4NDUwMzE3MzgyOCwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzEzMDczNzMwNDY4NzUsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNDM0NCJ9fSx7Im9yaWdpbiI6NCwid29yayI6eyJ0eXBlIjo4LCJzdWJ0eXBlIjp7Im9uZW9mS2luZCI6ImFydGljbGVUeXBlIiwiYXJ0aWNsZVR5cGUiOjF9LCJjb250cmlidXRvcnMiOlt7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJOYXZ5YSBWaWtyYW1hbiIsImZhbWlseSI6Ik5haXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUHJhdGVlcCBLdW1hciIsImZhbWlseSI6Ik5heWFrIn19XSwiaWRzIjp7Imlzc24iOlsiMjI5Ni03NzQ1Il0sImlzYm4iOltdLCJkb2kiOiIxMC4zMzg5L2ZtYXJzLjIwMjMuMTA4NzI5NiIsIm9wZW5hbGV4IjoiVzQzODI3MjMxMzMifSwidGl0bGUiOiJVbmNvdmVyaW5nIHdhdGVyIHF1YWxpdHkgYW5kIGV2YWx1YXRpbmcgdnVsbmVyYWJpbGl0aWVzIG9mIHNtYWxsLXNjYWxlIGZpc2hlcmllcyBpbiBDaGlsaWthIExhZ29vbiwgSW5kaWEiLCJhYnN0cmFjdCI6IlNtYWxsLXNjYWxlIGZpc2hlcmllcyAoU1NGcykgcGxheSBhIHN0cm9uZyByb2xlIGluIHN1c3RhaW5pbmcgbWlsbGlvbnMgb2YgbGl2ZWxpaG9vZHMsIGZvb2Qgc2VjdXJpdHksIG51dHJpdGlvbiwgYW5kIGluY29tZSBnbG9iYWxseSBidXQgdGhlIGZpc2hlcnMgZW5nYWdlZCBpbiB0aGlzIHNlY3RvciBzaW11bHRhbmVvdXNseSBleHBlcmllbmNlIGhpZ2ggbGV2ZWxzIG9mIHZ1bG5lcmFiaWxpdHkgYW5kIHByb2Nlc3NlcyBvZiBtYXJnaW5hbGlzYXRpb24uIFNldmVyYWwgZmFjdG9ycyBhcmUgYXR0cmlidXRlZCB0byB0aGUgbXVsdGlkaW1lbnNpb25hbCB2dWxuZXJhYmlsaXRpZXMgdGhlIHNtYWxsLXNjYWxlIGZpc2hlcnMgZXhwZXJpZW5jZSwgc3Bhbm5pbmcgYm90aCBuYXR1cmFsIChlLmcuLCBuYXR1cmFsIGRpc2FzdGVycywgZWNvc3lzdGVtIGNoYW5nZSkgYW5kIGFudGhyb3BvZ2VuaWMgKGUuZy4sIHBvbGljeSBjaGFuZ2UsIGh5ZHJvbG9naWNhbCBpbnRlcnZlbnRpb25zLCBhcXVhY3VsdHVyZSkgcHJlc3N1cmVzLiBXaGlsZSB0aGVyZSBpcyBtdWNoIGxpdGVyYXR1cmUgb24gdmFyaW91cyBuYXR1cmFsIGFuZCBodW1hbiBkcml2ZXJzIG9mIHZ1bG5lcmFiaWxpdHkgaW4gc21hbGwtc2NhbGUgZmlzaGVyeSBjb21tdW5pdGllcywgYW4gYWJzZW5jZSBvZiByZXNlYXJjaCBjb25uZWN0aW5nIHZ1bG5lcmFiaWxpdHkgd2l0aCB3YXRlciBxdWFsaXR5IGlzIGV2aWRlbnQuIEZpc2hlciBjb21tdW5pdGllcyBvZnRlbiB0YWxrIGFib3V0IGZpc2ggaW4gcmVsYXRpb24gdG8gdGhlIGhlYWx0aCBvZiB0aGVpciBhcXVhdGljIGhhYml0YXRzIHdoZXJlaW4gd2F0ZXIgcXVhbGl0eSBpcyBzZWVuIGFzIGEga2V5IHBhcmFtZXRlci4gVGhlIGxpbmsgYmV0d2VlbiBoZWFsdGh5IGZpc2ggYW5kIGdvb2QgcXVhbGl0eSB3YXRlciBoYXMgc2lnbmlmaWNhbnQgaW1wbGljYXRpb25zIGZvciBzdHJvbmcgYW5kIHZpYWJsZSBmaXNoaW5nIGNvbW11bml0aWVzLiBUaGlzIHBhcGVyIGV4YW1pbmVzIHRoZXNlIGxpbmtzIGZ1cnRoZXIgYnkgZm9jdXNpbmcgb24gdGhlIG5hdHVyZSBvZiB2dWxuZXJhYmlsaXRpZXMgY2F1c2VkIGJ5IHdhdGVyIHF1YWxpdHkgY2hhbmdlcyBpbiB0aGUgc21hbGwtc2NhbGUgZmlzaGVyeSBzeXN0ZW0gb2YgQ2hpbGlrYSBMYWdvb24gaW4gSW5kaWEuIFdlIHVuZGVydGFrZSBkZXRhaWxlZCBhbmFseXNpcyBvZiB0aGUgaW52YXNpdmUgc2hyaW1wIGFxdWFjdWx0dXJlIGFjdGl2aXRpZXMgYW5kIGh5ZHJvbG9naWNhbCBpbnRlcnZlbnRpb25zIGZvciBvcGVuaW5nIG9mIGEgbGFnb29uIGlubGV0IHdpdGggdGhlIEJheSBvZiBCZW5nYWwgYXMgdHdvIGRvbWluYW50IGRyaXZlcnMgYWR2ZXJzZWx5IGltcGFjdGluZyB3YXRlciBxdWFsaXR5IGFuZCBpbmNyZWFzaW5nIHZ1bG5lcmFiaWxpdGllcyBvZiB0aGUgZW50aXJlIHNtYWxsLXNjYWxlIGZpc2hlcmllcyBzb2NpYWwtZWNvbG9naWNhbCBzeXN0ZW0uIE91ciBhbmFseXNpcyBzdWdnZXN0cyB0aGF0IHRoZXJlIGFyZSBzdHJvbmcgaW50ZXJjb25uZWN0aW9ucyBiZXR3ZWVuIGNoYW5nZXMgaW4gd2F0ZXIgcXVhbGl0eSBhbmQgdGhlIGxldmVscyBvZiB2dWxuZXJhYmlsaXRpZXMgaW4gdGhlIFNTRnMgb2YgQ2hpbGlrYSBMYWdvb24uIFBvbGx1dGFudHMgc3VjaCBhcyBwZXN0aWNpZGVzLCBhbmQgb3JnYW5pYyBjb21wb3VuZHMgYWNjdW11bGF0ZSBpbiBmaXNoIHRpc3N1ZXMgYW5kIGFmZmVjdCB0aGVpciBncm93dGgsIHJlcHJvZHVjdGlvbiwgYW5kIG92ZXJhbGwgaGVhbHRoLiBUaGlzIGxlZCB0byBkZWNsaW5lcyBpbiBmaXNoIHBvcHVsYXRpb25zLCBtYWtpbmcgaXQgbW9yZSBkaWZmaWN1bHQgZm9yIGZpc2hlcnMgdG8gbWFrZSBhIGxpdmluZy4gSW4gYWRkaXRpb24gdG8gZGlyZWN0IGltcGFjdHMgb24gZmlzaCBwb3B1bGF0aW9ucywgcG9vciB3YXRlciBxdWFsaXR5IGFsc28gaGFzIGluZGlyZWN0IGVmZmVjdHMgb24gdGhlIHNvY2lhbCBhbmQgZWNvbm9taWMgdnVsbmVyYWJpbGl0eSBvZiBTU0ZzLiBGb3IgZXhhbXBsZSwgY29udGFtaW5hdGlvbiBvZiB3YXRlciBzb3VyY2VzIGxlZCB0byB0aGUgcmVkdWNlZCBudW1iZXIgb2YgZmlzaCBzcGVjaWVzIHJlZHVjaW5nIHRoZSBhbW91bnQgb2YgdGltZSBmaXNoZXJzIGNhbiBzcGVuZCBvbiBmaXNoaW5nIGFjdGl2aXRpZXMuIFRoaXMgYWxzbyBhZmZlY3RlZCB0aGUgbWFya2V0YWJpbGl0eSBvZiBmaXNoIHByb2R1Y3RzLCByZWR1Y2VkIGluY29tZSBhbmQgaW5jcmVhc2VkIHBvdmVydHkuIFRvIGZ1bGx5IHVuZGVyc3RhbmQgdGhlIGludGVyY29ubmVjdGlvbnMgYmV0d2VlbiB3YXRlciBxdWFsaXR5IGFuZCB2dWxuZXJhYmlsaXR5IGluIFNTRnMgaW4gQ2hpbGlrYSBMYWdvb24sIGl0IGlzIGltcG9ydGFudCB0byBjb25zaWRlciBib3RoIGVudmlyb25tZW50YWwgYW5kIHNvY2lhbCBmYWN0b3JzLCBhcyB3ZWxsIGFzIHRoZSBjb21wbGV4IGZlZWRiYWNrIGxvb3BzIGJldHdlZW4gdGhlc2UgZmFjdG9ycy4gVGhlIHN0dWR5IGhlbHBzIGluIGJyaWRnaW5nIGEgY3J1Y2lhbCBnYXAgaW4gb3VyIHVuZGVyc3RhbmRpbmcgb2YgdGhlIHJvbGUgb2Ygd2F0ZXIgcXVhbGl0eSBpbiB2dWxuZXJhYmlsaXR5IGFuYWx5c2lzIHdpdGhpbiByZXNvdXJjZSBkZXBlbmRlbnQgY29tbXVuaXRpZXMuIFdlIGNvbmNsdWRlIHdpdGgga2V5IGluc2lnaHRzIG9uIHBvc3NpYmxlIGNvcGluZyByZXNwb25zZXMgYW5kIGFkYXB0aXZlIGNhcGFjaXR5IG5lY2Vzc2FyeSBmb3IgdGhlIHNtYWxsLXNjYWxlIGZpc2hlcmllcyBjb21tdW5pdGllcyB0byB0cmFuc2l0aW9uIHRvd2FyZCB2aWFiaWxpdHkuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjMzODkvZm1hcnMuMjAyMy4xMDg3Mjk2IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5mcm9udGllcnNpbi5vcmcvYXJ0aWNsZXMvMTAuMzM4OS9mbWFycy4yMDIzLjEwODcyOTYvcGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjMtMDYtMjkiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTEwIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJGcm9udGllcnMgaW4gTWFyaW5lIFNjaWVuY2UiLCJ2b2x1bWUiOjEwLCJpbXBhY3RGYWN0b3IiOiI0LjI4In0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkZyb250aWVycyBNZWRpYSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NTEsImNpdGVkQnlDb3VudCI6NX19LCJxdW90ZSI6IldhdGVyIHF1YWxpdHkgb2YgQ2hpbGlrYSBMYWdvb24gaGFzIGRlZ3JhZGVkIGR1ZSB0byBhbnRocm9wb2dlbmljIGludGVydmVudGlvbnMgYW5kIG5hdHVyYWwgZm9yY2VzIG9mIGNoYW5nZS4gVmFyaW91cyBhY3Rpdml0aWVzIGluIHRoZSBDaGlsaWthIExhZ29vbiwgc3VjaCBhcyBpbmR1c3RyaWFsIHdhc3Rld2F0ZXIgZGlzcG9zYWwsIHNld2FnZSBkdW1waW5nLCBhcXVhY3VsdHVyZSwgaHlkcm9sb2dpY2FsIGludGVydmVudGlvbnMsIGFuZCBjeWNsb25lcywgYXJlIHByb2R1Y2luZyBzYWxpbml0eSBjaGFuZ2VzLCBzZWRpbWVudCBkZXBvc2l0aW9uLCBudXRyaWVudCBlbnJpY2htZW50LCBldXRyb3BoaWNhdGlvbiwgYW5kIGRlYWQgem9uZXMsIGFzIHN0YXRlZC4gRmlzaCBzdXJ2aXZhbCBpcyBhdCByaXNrIGFzIGEgcmVzdWx0IG9mIHRoZSBzaWduaWZpY2FudCByZWR1Y3Rpb24gaW4gZW52aXJvbm1lbnRhbCBjaXJjdW1zdGFuY2VzLCB3aGljaCBkaW1pbmlzaGVzIGZpc2hlcm1lbuKAmXMgaW5jb21lIGFuZCBwdXNoZXMgdGhlbSBvdXQgb2Ygc29jaWV0eS4gVGhlc2UgY2F1c2VzIGFmZmVjdGluZyB0aGUgd2F0ZXIgcXVhbGl0eSBvZiB0aGUgTGFnb29uIGRlcGljdCBtYW55IGZhY2VzIG9mIHZ1bG5lcmFiaWxpdHkgaW4gU1NGIGNvbW11bml0aWVzLiBBcyB0aGUgQ2hpbGlrYSBMYWdvb24gZW52aXJvbm1lbnQgaXMgZXhwZWN0ZWQgdG8gdW5kZXJnbyBpbmNyZWFzaW5nbHkgZHJhc3RpYyBjaGFuZ2VzIChzdWNoIGFzIGNhdGNoIGNvbXBvc2l0aW9uLCBjYXRjaCBjYXBhY2l0eSwgYmlvZGl2ZXJzaXR5IHZhcmlhdGlvbnMsIHdhdGVyIHF1YWxpdHksIGFuZCBmaXNoZXJ5IHJldmVudWVzKSwgaW50ZXJuYXRpb25hbCBwb2xpY3kgaGFzIGJlY29tZSBtb3JlIGludGVyZXN0ZWQgaW4gYWRhcHRhdGlvbiBtZWFzdXJlcy4gR2l2ZW4gaG93IGRpdmVyc2UgU1NGIGVjb3N5c3RlbXMgYXJlIGV4cGVjdGVkIHRvIGJlIGJldHdlZW4gY29udGluZW50cywgdGhlcmUgaXMgbm8g4oCcb25lIHNpemUgZml0cyBhbGzigJ0gYXBwcm9hY2guIFRoZSBwYXBlciBvdXRsaW5lZCBzZXZlcmFsIGNvcGluZyBhbmQgYWRhcHRhdGlvbiBtZWNoYW5pc21zIHRvIGltcHJvdmUgd2F0ZXIgcXVhbGl0eSBhbmQgY29udGludWUgZmlzaGluZyBpbiBhIHN1c3RhaW5hYmxlIHdheS4gQWRhcHRhdGlvbiBzdHJhdGVnaWVzIGNhbiBvY2Nhc2lvbmFsbHkgaGF2ZSB1bmZvcmVzZWVuIGNvbnNlcXVlbmNlcyBmb3IgZmlzaGVyaWVzIGFuZCBmaXNoaW5nIGNvbW11bml0aWVzLiBBbnkgdW5leHBlY3RlZCBjb25zZXF1ZW5jZXMgb2YgYWRhcHRhdGlvbiBjYW4gYmUgbWl0aWdhdGVkIHRocm91Z2ggbG9uZy10ZXJtIHBsYW5uaW5nIGFuZCBpZGVudGlmaWNhdGlvbiBvZiBmdXR1cmUgb3B0aW9ucyB0aHJvdWdoIHNjZW5hcmlvIGFuYWx5c2lzLiBUaGUgZmluZGluZ3Mgb2YgdGhpcyBwYXBlciBjYW4gYmUgdXRpbGl6ZWQgdG8gZGV2ZWxvcCBwb2xpY2llcyB0byBwcm90ZWN0IHdhdGVyIHF1YWxpdHkgYW5kIFNTRnMgaW4gYSB2YXJpZXR5IG9mIGNvYXN0YWwgbGFnb29uIHNldHRpbmdzLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyMi4yODUwMzYwODcwMzYxMzMsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40OTMwNTM0MzYyNzkzLCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyMy43NzM1MTk1MTU5OTEyMSwibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ4MzM2NDEwNTIyNDYxLCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyNS4yNTU1ODQ3MTY3OTY4NzUsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40ODk1NTkxNzM1ODM5ODQsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjI2Ljc0NDA2ODE0NTc1MTk1MywibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjUwNjgwNTQxOTkyMTg3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjguMjI2MTMzMzQ2NTU3NjE3LCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDkwNDYzMjU2ODM1OTQsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjI5LjcxNDYxNjc3NTUxMjY5NSwibGVmdCI6OC4zMzE1OTI1NTk4MTQ0NTMsIndpZHRoIjo0MC40OTQwOTg2NjMzMzAwOCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MzEuMTk2NjgzODgzNjY2OTkyLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDk0MzkyMzk1MDE5NTMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjMyLjY4NTE2NTQwNTI3MzQ0LCJsZWZ0Ijo4LjMzMTU5NjM3NDUxMTcxOSwid2lkdGgiOjQwLjQ4NzQ0NTgzMTI5ODgzLCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozNC4xNjcyMzI1MTM0Mjc3MzQsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDk3NzQ1NTEzOTE2MDE2LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozNS42NTU3MTU5NDIzODI4MSwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40OTg2MzgxNTMwNzYxNywiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MzcuMTM3NzgzMDUwNTM3MTEsImxlZnQiOjEwLjY4NTE4NjM4NjEwODM5OCwid2lkdGgiOjM4LjE0MTEwNTY1MTg1NTQ3LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozOC42MjYyNjY0Nzk0OTIxOSwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40ODcxNTIwOTk2MDkzNzUsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQwLjEwODMzMzU4NzY0NjQ4NCwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40ODk4NTY3MTk5NzA3LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo0MS41OTY4MTMyMDE5MDQzLCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4ODY2NjUzNDQyMzgzLCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo0My4wNzg4ODAzMTAwNTg1OTQsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDgxODYxMTE0NTAxOTUsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQ0LjU2NzM2MzczOTAxMzY3LCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4MzA1ODkyOTQ0MzM2LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo0Ni4wNDk0MzA4NDcxNjc5NywibGVmdCI6OC4zMzMxMDg5MDE5Nzc1MzksIndpZHRoIjo0MC41MDA3NDc2ODA2NjQwNiwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDcuNTM3OTE0Mjc2MTIzMDUsImxlZnQiOjguMzMzMTA4OTAxOTc3NTM5LCJ3aWR0aCI6NDAuNDk1NjA1NDY4NzUsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQ5LjAxOTk4MTM4NDI3NzM0NCwibGVmdCI6OC4zMzMxMDYwNDA5NTQ1OSwid2lkdGgiOjQwLjUwNjgxNjg2NDAxMzY3LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo1MC41MDg0NjQ4MTMyMzI0MiwibGVmdCI6OC4zMzMxMDYwNDA5NTQ1OSwid2lkdGgiOjQwLjQ4ODUwNjMxNzEzODY3LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo1MS45OTA1MjgxMDY2ODk0NSwibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ5NzcyNjQ0MDQyOTY5LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo1My40NzkwMTE1MzU2NDQ1MywibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjMzLjE1OTQ0NjcxNjMwODU5NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NTQuOTYxMDc4NjQzNzk4ODMsImxlZnQiOjEwLjY4MzY3MDA0Mzk0NTMxMiwid2lkdGgiOjM4LjEzNjU5NjY3OTY4NzUsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjU2LjQ0OTU2MjA3Mjc1MzkwNiwibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ4NzE0NDQ3MDIxNDg0NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIxMyJ9fV0%3D. Such dynamics not only amplify biomass but also toxin production, with microcystins surging 3-fold in warmer conditions due to upregulated biosynthetic pathways (Gobler et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkNocmlzdG9waGVyIEouIiwiZmFtaWx5IjoiR29ibGVyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlJ1dGggVy4iLCJmYW1pbHkiOiJEcmlua3dhdGVyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkEuIFcuIiwiZmFtaWx5IjoiQW50aG9ueSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJKZW5uaWZlciBBLiIsImZhbWlseSI6IkdvbGVza2kifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW5uIE1hcmllIEUuIiwiZmFtaWx5IjoiRmFtdWxhcm8tUGVjb3JhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1pY2hhZWwgQi4iLCJmYW1pbHkiOiJXYWxsYWNlIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5vcmEgUi4gVy4iLCJmYW1pbHkiOiJTdHJhcXVhZGluZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJSb25vam95IiwiZmFtaWx5IjoiSGVtIn19XSwiaWRzIjp7Imlzc24iOlsiMTY2NC0zMDJYIl0sImlzYm4iOltdLCJkb2kiOiIxMC4zMzg5L2ZtaWNiLjIwMjQuMTQ2NDY4NiIsInBtaWQiOiIzOTU2ODk5NyIsIm9wZW5hbGV4IjoiVzQ0MDQxMjA5NTIiLCJleGEiOiJodHRwczovL3d3dy5mcm9udGllcnNpbi5vcmcvam91cm5hbHMvbWljcm9iaW9sb2d5L2FydGljbGVzLzEwLjMzODkvZm1pY2IuMjAyNC4xNDY0Njg2L2Z1bGwifSwidGl0bGUiOiJTZXdhZ2UtYW5kIGZlcnRpbGl6ZXItZGVyaXZlZCBudXRyaWVudHMgYWx0ZXIgdGhlIGludGVuc2l0eSwgZGl2ZXJzaXR5LCBhbmQgdG94aWNpdHkgb2YgaGFybWZ1bCBjeWFub2JhY3RlcmlhbCBibG9vbXMgaW4gZXV0cm9waGljIGxha2VzIiwiYWJzdHJhY3QiOiJDeWFub2JhY3RlcmlhbCBoYXJtZnVsIGFsZ2FsIGJsb29tcyAoQ0hBQnMpIGFyZSBwcm9tb3RlZCBieSBleGNlc3NpdmUgbnV0cmllbnQgbG9hZGluZyBhbmQsIHdoaWxlIGZlcnRpbGl6ZXJzIGFuZCBzZXdhZ2UgYXJlIHRoZSBtb3N0IHByZXZhbGVudCBleHRlcm5hbCBudXRyaWVudCBzb3VyY2VzIGluIG1vc3Qgd2F0ZXJzaGVkcywgdGhlIGRpZmZlcmVudGlhbCBlZmZlY3RzIG9mIHRoZXNlIG51dHJpZW50IHNvdXJjZXMgb24gQ0hBQnMgYXJlIHVua25vd24uIEhlcmUsIHdlIHRyYWNrZWQgQ0hBQnMgYW5kIHBlcmZvcm1lZCBleHBlcmltZW50cyBpbiBmaXZlIGRpc3RpbmN0IGxha2VzIGFjcm9zcyB0aGUgTm9ydGhlcm4gVVMgaW5jbHVkaW5nIExha2UgRXJpZS4gRmVydGlsaXplcnMgd2l0aCBhbW1vbml1bSBhbmQgb3J0aG9waG9zcGhhdGUsIG1lbWJyYW5lICgwLjIgzrxtKS1maWx0ZXJlZCBzZXdhZ2UgKGRvbWluYXRlZCBieSByZWR1Y2VkIGZvcm1zIG9mIG5pdHJvZ2VuKSBzYW5kLWFuZCBtZW1icmFuZS1maWx0ZXJlZCBzZXdhZ2UgKGRvbWluYXRlZCBieSBuaXRyYXRlKSwgYW5kIGFuIGlub3JnYW5pYyBudXRyaWVudCBzb2x1dGlvbiBvZiBhbW1vbml1bSBhbmQgb3J0aG9waG9zcGhhdGUgd2VyZSB1c2VkIGFzIGV4cGVyaW1lbnRhbCBudXRyaWVudCBzb3VyY2VzIGZvciBDSEFCcyBhdCBOLWVxdWl2YWxlbnQsIGVudmlyb25tZW50YWxseSByZWFsaXN0aWMgY29uY2VudHJhdGlvbnMuIFBoeXRvcGxhbmt0b24gY29tbXVuaXRpZXMgd2VyZSBldmFsdWF0ZWQgZmx1b3JvbWV0cmljYWxseSwgbWljcm9zY29waWNhbGx5LCBhbmQgdmlhIGhpZ2ggdGhyb3VnaHB1dCBzZXF1ZW5jaW5nIG9mIHRoZSAxNlMgclJOQSBnZW5lLCBhbmQgbGV2ZWxzIG9mIG1pY3JvY3lzdGluIGFuZCB0aGUgzrQgMTUgTiBjb250ZW50IG9mIHBhcnRpY3VsYXRlIG9yZ2FuaWMgbml0cm9nZW4gKM60UE8gMTUgTikgd2VyZSBxdWFudGlmaWVkLiBGZXJ0aWxpemVyIGFuZCBib3RoIHNvdXJjZXMgb2Ygd2FzdGV3YXRlciBpbmNyZWFzZWQgdGhlIGFidW5kYW5jZSBvZiBjeWFub2JhY3RlcmlhIGluIGFsbCBleHBlcmltZW50cyBhY3Jvc3MgYWxsIGZpdmUgbGFrZXMgKCBwICZhbXA7bHQ7IDAuMDUgZm9yIGFsbCkgd2hlcmVhcyBlZmZlY3RzIG9uIGV1a2FyeW90aWMgcGh5dG9wbGFua3RvbiB3ZXJlIGxpbWl0ZWQuIFNhbmQtZmlsdGVyZWQgc2V3YWdlIGNvbnRhaW5lZCBsZXNzIFAsIG9yZ2FuaWMgbWF0dGVyLCBhbmQgYW1tb25pdW0gYnV0IG1vcmUgbml0cmF0ZSBhbmQgaGFkIGEgMjUlIGxlc3MgcG90ZW50IHN0aW11bGF0b3J5IGVmZmVjdCBvbiBjeWFub2JhY3RlcmlhIHRoYW4gbWVtYnJhbmUtZmlsdGVyZWQgc2V3YWdlLCBzdWdnZXN0aW5nIG5pdHJpZmljYXRpb24gbWF5IHBsYXkgYSByb2xlIGluIHJlZHVjaW5nIENIQUJzLiBGZXJ0aWxpemVyIGluY3JlYXNlZCBtaWNyb2N5c3RpbiBsZXZlbHMgYW5kIGRlY3JlYXNlZCB0aGUgzrRQTyAxNSBOIHdoZXJlYXMgd2FzdGV3YXRlciBpbmNyZWFzZWQgzrRQTyAxNSBOICggcCAmYW1wO2x0OyAwLjA1IGZvciBhbGwpLiBNaWNyb2N5c3RpcyB3YXMgdGhlIGdlbnVzIG1vc3QgY29uc2lzdGVudGx5IHByb21vdGVkIGJ5IG51dHJpZW50IHNvdXJjZXMgKCBwICZhbXA7bHQ7IDAuMDUgaW4gYWxsIGV4cGVyaW1lbnRzKSwgZm9sbG93ZWQgYnkgQ3lhbm9iaXVtICggcCAmYW1wO2x0OyAwLjA1IGluIDUwJSBvZiBleHBlcmltZW50cyksIHdpdGggaW5jcmVhc2VzIGluIE1pY3JvY3lzdGlzIGJpb21hc3MgY29uc2lzdGVudGx5IGVsaWNpdGVkIGJ5IG1lbWJyYW5lLWZpbHRlcmVkIHdhc3Rld2F0ZXIuIENvbGxlY3RpdmVseSwgcmVzdWx0cyBkZW1vbnN0cmF0ZSB0aGF0IGRpZmZlcmluZyB0eXBlcyBvZiBzZXdhZ2UgZGlzY2hhcmdlIGFuZCBmZXJ0aWxpemVycyBjYW4gcHJvbW90ZSBDSEFCIGludGVuc2l0eSBhbmQgdG94aWNpdHksIHdoaWxlIGNvbmN1cnJlbnRseSBhbHRlcmluZyBDSEFCIGRpdmVyc2l0eSBhbmQgzrRQTyAxNSBOLiBXaGlsZSBtZW1icmFuZS1maWx0ZXJlZCBzZXdhZ2UgY29uc2lzdGVudGx5IGZhdm9yZWQgTWljcm9jeXN0aXMgLCB0aGUgZGlzY2hhcmdlIG9mIHNld2FnZSB0aHJvdWdoIHNhbmRzIG11dGVkIGJsb29tIGludGVuc2l0eSBzdWdnZXN0aW5nIHNhbmQtYmVkcyBtYXkgcmVwcmVzZW50IGEgdG9vbCB0byByZW1vdmUga2V5IG51dHJpZW50cyBhbmQgcGFydGlhbGx5IG1pdGlnYXRlIENIQUJzLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzg5L2ZtaWNiLjIwMjQuMTQ2NDY4NiIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cuZnJvbnRpZXJzaW4ub3JnL2pvdXJuYWxzL21pY3JvYmlvbG9neS9hcnRpY2xlcy8xMC4zMzg5L2ZtaWNiLjIwMjQuMTQ2NDY4Ni9wZGYiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMS0wNiIsImFjY2Vzc2VkRGF0ZSI6IjIwMjYtMDIiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZyb250aWVycyBpbiBNaWNyb2Jpb2xvZ3kiLCJ2b2x1bWUiOjE1LCJpbXBhY3RGYWN0b3IiOiIzLjk5In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTQ2NDY4NiwibGFzdFBhZ2UiOjE0NjQ2ODZ9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRnJvbnRpZXJzIE1lZGlhIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo3MywiY2l0ZWRCeUNvdW50Ijo5fX0sInF1b3RlIjoiZGVyaXZlZCBudXRyaWVudHMgYWx0ZXIgdGhlIGludGVuc2l0eSwgZGl2ZXJzaXR5LCBhbmQgdG94aWNpdHkgb2YgaGFybWZ1bCBjeWFub2JhY3RlcmlhbCBibG9vbXMgLi4uIEN5YW5vYmFjdGVyaWFsIGhhcm1mdWwgYWxnYWwgYmxvb21zIChDSEFCcykgYXJlIHByb21vdGVkIGJ5IGV4Y2Vzc2l2ZSBudXRyaWVudCBsb2FkaW5nIGFuZCwgd2hpbGUgZmVydGlsaXplcnMgYW5kIHNld2FnZSBhcmUgdGhlIG1vc3QgcHJldmFsZW50IGV4dGVybmFsIG51dHJpZW50IHNvdXJjZXMgaW4gbW9zdCB3YXRlcnNoZWRzLCB0aGUgZGlmZmVyZW50aWFsIGVmZmVjdHMgb2YgdGhlc2UgbnV0cmllbnQgLi4uIG9yZ2FuaWMgbml0cm9nZW4gKM60UE8xNU4pIHdlcmUgcXVhbnRpZmllZC4gRmVydGlsaXplciBhbmQgYm90aCBzb3VyY2VzIG9mIHdhc3Rld2F0ZXIgaW5jcmVhc2VkIHRoZSBhYnVuZGFuY2Ugb2YgLi4uIHBoeXRvcGxhbmt0b24gd2VyZSBsaW1pdGVkLiBTYW5kLWZpbHRlcmVkIHNld2FnZSBjb250YWluZWQgbGVzcyBQLCBvcmdhbmljIG1hdHRlciwgYW5kIGFtbW9uaXVtIGJ1dCBtb3JlIG5pdHJhdGUgYW5kIC4uLiBkaWZmZXJpbmcgdHlwZXMgb2Ygc2V3YWdlIGRpc2NoYXJnZSBhbmQgZmVydGlsaXplcnMgY2FuIHByb21vdGUgQ0hBQiBpbnRlbnNpdHkgYW5kIHRveGljaXR5LCB3aGlsZSBjb25jdXJyZW50bHkgLi4uIHNld2FnZSB0aHJvdWdoIHNhbmRzIG11dGVkIGJsb29tIGludGVuc2l0eSBzdWdnZXN0aW5nIHNhbmQtYmVkcyBtYXkgcmVwcmVzZW50IGEgdG9vbCB0byByZW1vdmUga2V5IG51dHJpZW50cyBhbmQgLi4uICggV2F0c29uIGV0IGFsLiwgMjAxNiA7IEJ1bGxlcmphaG4gZXQgYWwuLCAyMDE2IDsgSHVpc21hbiBldCBhbC4sIDIwMTggKSBhbmQgZXhjZXNzaXZlIG51dHJpZW50IGxvYWRpbmcgaXMgdGhlIHByb2Nlc3MgbW9zdCBjb21tb25seSBjaXRlZCBhcyB0aGUgZW52aXJvbm1lbnRhbCBmYWN0b3IgcHJvbW90aW5nIENIQUJzICgifV0%3D.

Altered monsoon cycles compound these effects by disrupting nutrient flushing and hydrological retention. India's monsoons, vital for diluting pollutants, now exhibit bimodal patterns: extreme early floods erode riparian soils, delivering pulsed N and P loads, followed by deficits that halve lake volumes and quadruple nutrient concentrations (Sitote & Gebremedhine, 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllhYnNyYSBNZWxhayIsImZhbWlseSI6IlNpdG90ZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNaWNoYWVsIEdpcmltYXkiLCJmYW1pbHkiOiJHZWJyZW1lZGhpbmUifX1dLCJpZHMiOnsiaXNzbiI6W10sImlzYm4iOltdLCJkb2kiOiIxMC4yMDk0NC9wcmVwcmludHMyMDI0MDkuMDcwNC52MSIsIm9wZW5hbGV4IjoiVzQ0MDI0NjU0MTQiLCJleGEiOiJodHRwczovL3d3dy5wcmVwcmludHMub3JnL21hbnVzY3JpcHQvMjAyNDA5LjA3MDQifSwidGl0bGUiOiJDb21wcmVoZW5zaXZlIFJldmlldyBvZiBFdXRyb3BoaWNhdGlvbiBpbiBGcmVzaHdhdGVyIEVjb3N5c3RlbXM6IENhdXNlcywgRWZmZWN0cywgQXNzZXNzbWVudCwgYW5kIE1hbmFnZW1lbnQgU3RyYXRlZ2llcyIsImFic3RyYWN0IjoiVGhpcyByZXZpZXcgZXhhbWluZXMgZXV0cm9waGljYXRpb24gaW4gZnJlc2h3YXRlciBlY29zeXN0ZW1zLCBhIHdpZGVzcHJlYWQgZW52aXJvbm1lbnRhbCBpc3N1ZSBzdGVtbWluZyBmcm9tIGV4Y2Vzc2l2ZSBudXRyaWVudCBpbnB1dCwgbGFyZ2VseSBkdWUgdG8gaHVtYW4gYWN0aXZpdGllcy4gVGhlIHN0dWR5IGV4cGxvcmVzIHRoZSBvcmlnaW5zLCBjb25zZXF1ZW5jZXMsIGV2YWx1YXRpb24gdGVjaG5pcXVlcywgYW5kIGNvbnRyb2wgbWV0aG9kcyBmb3IgZXV0cm9waGljYXRpb24uIFByaW1hcnkgY2F1c2VzIGluY2x1ZGUgYWdyaWN1bHR1cmFsIGRpc2NoYXJnZSwgdXJiYW4gZGV2ZWxvcG1lbnQsIGFuZCBjbGltYXRlIHNoaWZ0cy4gVGhlIGVmZmVjdHMgYXJlIGZhci1yZWFjaGluZywgaW1wYWN0aW5nIGJpb2RpdmVyc2l0eSwgd2F0ZXIgcXVhbGl0eSwgYW5kIHNvY2lvLWVjb25vbWljIGZhY3RvcnMuIFRoZSBhc3Nlc3NtZW50IHV0aWxpemVzIGEgY29tYmluYXRpb24gb2YgY2hlbWljYWwsIHBoeXNpY2FsLCBhbmQgYmlvbG9naWNhbCBtYXJrZXJzLCB3aXRoIGN1dHRpbmctZWRnZSB0ZWNobm9sb2dpZXMgbGlrZSByZW1vdGUgc2Vuc2luZyBpbXByb3ZpbmcgbW9uaXRvcmluZyBlZmZvcnRzLiBNaXRpZ2F0aW9uIHN0cmF0ZWdpZXMgYXJlIGRpdmVyc2UsIGluY29ycG9yYXRpbmcgcHJldmVudGF0aXZlIGFjdGlvbnMgKHN1Y2ggYXMgbnV0cmllbnQgbGltaXRhdGlvbiBhbmQgbGFuZCBtYW5hZ2VtZW50KSwgY29ycmVjdGl2ZSBtZWFzdXJlcyAoaW5jbHVkaW5nIGJpby1tYW5pcHVsYXRpb24gYW5kIGNoZW1pY2FsIGFwcGxpY2F0aW9ucyksIGFuZCByZWd1bGF0b3J5IGluaXRpYXRpdmVzIChsaWtlIHdhdGVyIHF1YWxpdHkgZ3VpZGVsaW5lcyBhbmQgbnV0cmllbnQgZXhjaGFuZ2Ugc2NoZW1lcykuIFdoaWxlIGNhc2Ugc3R1ZGllcyBkZW1vbnN0cmF0ZSBzdWNjZXNzZnVsIGludGVydmVudGlvbnMsIHRoZXkgYWxzbyByZXZlYWwgb25nb2luZyBjaGFsbGVuZ2VzLCBwYXJ0aWN1bGFybHkgZWNvc3lzdGVtIHVucHJlZGljdGFiaWxpdHkgYW5kIGNsaW1hdGUgY2hhbmdlIGluZmx1ZW5jZXMuIFRoZSByZXNlYXJjaCBoaWdobGlnaHRzIHRoZSBwcm9taXNlIG9mIGVtZXJnaW5nIHRlY2hub2xvZ2llcywgbm90YWJseSBtYWNoaW5lIGxlYXJuaW5nLCBpbiBlbmhhbmNpbmcgbWFuYWdlbWVudCBwcmFjdGljZXMuIEtub3dsZWRnZSBnYXBzIHBlcnNpc3QsIGVzcGVjaWFsbHkgcmVnYXJkaW5nIGV1dHJvcGhpY2F0aW9uLWJpb2RpdmVyc2l0eSBkeW5hbWljcyBpbiB0cm9waWNhbCBlbnZpcm9ubWVudHMgYW5kIHRoZSBzb2Npby1lY29ub21pYyByYW1pZmljYXRpb25zIG9mIGludGVydmVudGlvbnMuIFRoZSBzdHVkeSBlbXBoYXNpemVzIHRoZSBpbXBvcnRhbmNlIG9mIGZsZXhpYmxlLCBjb21wcmVoZW5zaXZlIGFwcHJvYWNoZXMgdGhhdCBibGVuZCBzY2llbnRpZmljIGluc2lnaHRzIHdpdGggcHJhY3RpY2FsIHNvbHV0aW9ucyBhbmQgc3Ryb25nIHBvbGljeSBzdHJ1Y3R1cmVzIHRvIHRhY2tsZSB0aGlzIGludHJpY2F0ZSBlbnZpcm9ubWVudGFsIGNoYWxsZW5nZSBpbiBhbiBlcmEgb2YgY2xpbWF0ZSBjaGFuZ2UuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjIwOTQ0L3ByZXByaW50czIwMjQwOS4wNzA0LnYxIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjA5NDQvcHJlcHJpbnRzMjAyNDA5LjA3MDQudjEiLCJpc3N1ZWREYXRlIjoiMjAyNC0wOS0xMCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTEiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IlByZXByaW50cy5vcmciLCJpbXBhY3RGYWN0b3IiOiIxLjE1In0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjowLCJjaXRlZEJ5Q291bnQiOjR9fSwicXVvdGUiOiJUaGUgc3R1ZHkgZW1waGFzaXplcyB0aGUgaW1wb3J0YW5jZSBvZiBmbGV4aWJsZSwgY29tcHJlaGVuc2l2ZSBhcHByb2FjaGVzIHRoYXQgYmxlbmQgc2NpZW50aWZpYyBpbnNpZ2h0cyB3aXRoIHByYWN0aWNhbCBzb2x1dGlvbnMgYW5kIHN0cm9uZyBwb2xpY3kgc3RydWN0dXJlcyB0byB0YWNrbGUgdGhpcyBpbnRyaWNhdGUgZW52aXJvbm1lbnRhbCBjaGFsbGVuZ2UgaW4gYW4gZXJhIG9mIGNsaW1hdGUgY2hhbmdlLiBFdXRyb3BoaWNhdGlvbiwgY2F1c2VkIGJ5IGV4Y2Vzc2l2ZSBudXRyaWVudCBlbnJpY2htZW50IGluIHdhdGVyIGJvZGllcywgaXMgYSBzaWduaWZpY2FudCBlbnZpcm9ubWVudGFsIGNoYWxsZW5nZSBmb3IgZnJlc2h3YXRlciBlY29zeXN0ZW1zIG9uIGEgZ2xvYmFsIHNjYWxlLiBQcmltYXJpbHkgZHJpdmVuIGJ5IG5pdHJvZ2VuIGFuZCBwaG9zcGhvcnVzIGlucHV0cywgdGhpcyBwcm9jZXNzIHJlc3VsdHMgaW4gdGhlIG92ZXJwcm9kdWN0aW9uIG9mIGFsZ2FlIGFuZCBhcXVhdGljIHBsYW50cywgbGVhZGluZyB0byBhIGNhc2NhZGUgb2YgZWNvbG9naWNhbCBkaXN0dXJiYW5jZXMsIGluY2x1ZGluZyBveHlnZW4gZGVwbGV0aW9uIChoeXBveGlhKSwgbG9zcyBvZiBiaW9kaXZlcnNpdHksIGFuZCBzZXZlcmUgd2F0ZXIgcXVhbGl0eSBkZWdyYWRhdGlvbiAoS2FrYWRlIGV0IGFsLiwgMjAyMTsgTWVlcmhvZmYgZXQgYWwuLCAyMDIyOyBXdXJ0c2JhdWdoIGV0IGFsLiwgMjAxOSkuIn1d. This hydrological volatility selects for resilient, bloom-forming genera like Planktothrix, which tolerate fluctuating salinities and temperatures akin to industrial effluents (Paerl, 2018)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhhbnMgVy4iLCJmYW1pbHkiOiJQYWVybCJ9fV0sImlkcyI6eyJpc3NuIjpbXSwiaXNibiI6W10sImRvaSI6IjEwLjE3NjE1L3luNTUtYjQzOCIsIm9wZW5hbGV4IjoiVzQzMDcwNjM3MzkifSwidGl0bGUiOiJNaXRpZ2F0aW5nIHRveGljIHBsYW5rdG9uaWMgY3lhbm9iYWN0ZXJpYWwgYmxvb21zIGluIGFxdWF0aWMgZWNvc3lzdGVtcyBmYWNpbmcgaW5jcmVhc2luZyBhbnRocm9wb2dlbmljIGFuZCBjbGltYXRpYyBwcmVzc3VyZXMiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTc2MTUveW41NS1iNDM4IiwicmVzb3VyY2VVcmwiOiJodHRwczovL2Nkci5saWIudW5jLmVkdS9kb3dubG9hZHMvd2gyNDcyOTBnIiwiaXNzdWVkRGF0ZSI6IjIwMTgtMDEtMDEiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA2IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJDYXJvbGluYSBEaWdpdGFsIFJlcG9zaXRvcnkgKFVuaXZlcnNpdHkgb2YgTm9ydGggQ2Fyb2xpbmEgYXQgQ2hhcGVsIEhpbGwpIiwiaW1wYWN0RmFjdG9yIjoiMC4xMCJ9LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJVbml2ZXJzaXR5IG9mIE5vcnRoIENhcm9saW5hIGF0IENoYXBlbCBIaWxsIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjowLCJjaXRlZEJ5Q291bnQiOjB9fSwicXVvdGUiOiJUaGlzIGxlYWRzIHRvIGNvbnRpbnVpbmcgZGVtYW5kIGZvciBOIHN1cHBsaWVzIHRvIHN1c3RhaW4gcHJpbWFyeSBwcm9kdWN0aW9uIGFuZCBwcm9tb3RlIGV1dHJvcGhpY2F0aW9uIFszOCwzOV0sICgyKSBhIGxlbmd0aHkgbGVnYWN5IG9mIFAgbG9hZGluZyB0aGF0IGlzIHN0b3JlZCBpbiBzZWRpbWVudHMsIHdoaWNoIGlzIHJlYWRpbHkgcmVjeWNsZWQgYW5kIHN1cHBvcnRzIGV1dHJvcGhpY2F0aW9uLCAoMykgTjIgZml4YXRpb24sIHdoaWxlIHByb3ZpZGluZyDigJxuZXfigJ0gTiwgZmFpbHMgdG8gbWVldCBlY29zeXN0ZW0tc2NhbGUgTiByZXF1aXJlbWVudHMgWzQwLDQxXS4gRnJvbSBhIG51dHJpZW50IG1hbmFnZW1lbnQgcGVyc3BlY3RpdmUsIHRoaXMgbWVhbnMgdGhhdCBjb250aW51ZWQgZXh0ZXJuYWwgbG9hZGluZyBvZiBOIHBsYXlzIGEgY3JpdGljYWwgcm9sZSBpbiBhY2NlbGVyYXRpbmcgZXV0cm9waGljYXRpb24gYW5kIHByb21vdGluZyBhbGdhbCBibG9vbXMgWzM2XS4gT25lIHRyb3VibGluZyBpbmRpY2F0b3Igb2YgTi1kcml2ZW4gZXV0cm9waGljYXRpb24gYW5kIEN5YW5vSEFCcyBpbiBmcmVzaHdhdGVyIHN5c3RlbXMgaXMgdGhlIGdlb2dyYXBoaWMgZXhwYW5zaW9uIGFuZCBpbmNyZWFzZSBmcmVxdWVuY3kgb2YgdGhlIG5vbi1OMiBmaXhpbmcgdG94aW4tcHJvZHVjaW5nIGdlbnVzIE1pY3JvY3lzdGlzIFs0Ml0sIGVzcGVjaWFsbHkgaW4gbGFyZ2UgbGFrZXMgKGUuZy4sIExha2UgRXJpZSwgQ2FuYWRhLVVTQSwgTGFrZSBLYXN1bWFndXJhLCBKYXBhbiwgTGFrZSBPa2VlY2hvYmVlLCBGbG9yaWRhLCBVU0EsIExha2UgVGFpaHUsIENoaW5hKS4gVGhpcyBnZW51cywgYXMgd2VsbCBhcyBhbm90aGVyIHBsYW5rdG9uaWMgbm9uLU4yIGZpeGluZyBDeWFub0hBQiBQbGFua3RvdGhyaXgsIGFuZCBzZXZlcmFsIGJlbnRoaWMgYW5hbG9ndWUgYmxvb20gdGF4YSAoZS5nLiwgbm9uLU4yIGZpeGluZyBMeW5nYnlhIGFuZCBPc2NpbGxhdG9yaWEgc3BwLiksIHNlcnZlIGFzIGluZGljYXRvcnMgb2YgTi1vdmVyLWVucmljaG1lbnQgWzM2LDQyXS4gVGhlIHByb2xpZmVyYXRpb24gb2YgdGhlc2Ugbm9uLU4yIGZpeGluZyBDeWFub0hBQnMgaXMgYW4gYWRkaXRpb25hbCBpbmRpY2F0b3IgdGhlIE4gaW5wdXQgcmVkdWN0aW9ucyBhcmUgbmVlZGVkIHRvIHJldmVyc2UgdGhpcyB0cmVuZC4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo3Ny40NDYyNTA5MTU1MjczNCwibGVmdCI6MTYuNDI4NTE0NDgwNTkwODIsIndpZHRoIjo3MC43MTQ3MjE2Nzk2ODc1LCJoZWlnaHQiOjEuNDUwNzk2NDg0OTQ3MjA0Nn19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjc5LjA1MDk3OTYxNDI1NzgxLCJsZWZ0IjoxMi44NTcwNjEzODYxMDgzOTgsIndpZHRoIjo3NC40OTQ5ODc0ODc3OTI5NywiaGVpZ2h0IjoxLjQ1MDc5NjQ4NDk0NzIwNDZ9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo4MC42NTU1ODYyNDI2NzU3OCwibGVmdCI6MTIuODAxOTYxODk4ODAzNzExLCJ3aWR0aCI6NzQuMzQwMDE5MjI2MDc0MjIsImhlaWdodCI6MS40NTA3OTY0ODQ5NDcyMDQ2fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6ODIuMjYwMTg1MjQxNjk5MjIsImxlZnQiOjEyLjg1NzA2MTM4NjEwODM5OCwid2lkdGgiOjc0LjI4NTE4Njc2NzU3ODEyLCJoZWlnaHQiOjEuNjY2OTc3NjQzOTY2Njc0OH19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjgzLjg2NDkwNjMxMTAzNTE2LCJsZWZ0IjoxMi44NTcwNjEzODYxMDgzOTgsIndpZHRoIjo3NC4yODUwMTg5MjA4OTg0NCwiaGVpZ2h0IjoxLjQ1MDc5NjQ4NDk0NzIwNDZ9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo4NS40Njk1MTI5Mzk0NTMxMiwibGVmdCI6MTIuODU3MDYxMzg2MTA4Mzk4LCJ3aWR0aCI6NzEuMTA0OTg4MDk4MTQ0NTMsImhlaWdodCI6MS40NTA3OTY0ODQ5NDcyMDQ2fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6ODcuMDc0MjM0MDA4Nzg5MDYsImxlZnQiOjE2LjQyODUxNDQ4MDU5MDgyLCJ3aWR0aCI6NzAuNzE0MTcyMzYzMjgxMjUsImhlaWdodCI6MS40NTA3OTY0ODQ5NDcyMDQ2fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6ODguNjc4ODQwNjM3MjA3MDMsImxlZnQiOjEyLjg1NzA2MTM4NjEwODM5OCwid2lkdGgiOjc0LjI4NjAyNjAwMDk3NjU2LCJoZWlnaHQiOjEuNjY2OTc3NjQzOTY2Njc0OH19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjkwLjI4MzQzOTYzNjIzMDQ3LCJsZWZ0IjoxMi44NTcwNjEzODYxMDgzOTgsIndpZHRoIjo3NC4yODU0MTU2NDk0MTQwNiwiaGVpZ2h0IjoxLjQ3NjgzMDQ4MjQ4MjkxMDJ9fSx7InBhZ2VOdW1iZXIiOjYsImJib3giOnsidG9wIjoxMC41MDY1MTM1OTU1ODEwNTUsImxlZnQiOjEyLjg1NzA2MTM4NjEwODM5OCwid2lkdGgiOjc0LjIwMjAxODczNzc5Mjk3LCJoZWlnaHQiOjEuNTg2NjA4NDA5ODgxNTkxOH19LHsicGFnZU51bWJlciI6NiwiYmJveCI6eyJ0b3AiOjEyLjExMTIzNzUyNTkzOTk0MSwibGVmdCI6MTIuODU3MDYxMzg2MTA4Mzk4LCJ3aWR0aCI6NzQuMjg1Njc1MDQ4ODI4MTIsImhlaWdodCI6MS42NjY5ODEzMzk0NTQ2NTA5fX0seyJwYWdlTnVtYmVyIjo2LCJiYm94Ijp7InRvcCI6MTMuNzE1ODM3NDc4NjM3Njk1LCJsZWZ0IjoxMi44NTcwNjEzODYxMDgzOTgsIndpZHRoIjo3NC4yODU5NzI1OTUyMTQ4NCwiaGVpZ2h0IjoxLjQ3NjgzNDE3Nzk3MDg4NjJ9fSx7InBhZ2VOdW1iZXIiOjYsImJib3giOnsidG9wIjoxNS4zMjA1NjE0MDg5OTY1ODIsImxlZnQiOjEyLjg1NzA2MTM4NjEwODM5OCwid2lkdGgiOjc0LjI4NTEwMjg0NDIzODI4LCJoZWlnaHQiOjEuNjY2ODY1MzQ4ODE1OTE4fX0seyJwYWdlTnVtYmVyIjo2LCJiYm94Ijp7InRvcCI6MTYuOTI1MTYzMjY5MDQyOTcsImxlZnQiOjEyLjg1NzA2MTM4NjEwODM5OCwid2lkdGgiOjc0LjI4NTM1NDYxNDI1NzgxLCJoZWlnaHQiOjEuNjY2OTgxMzM5NDU0NjUwOX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjYifX1d. In urban lakes encircled by concretized catchments, reduced infiltration prolongs retention times, allowing accumulated heat to sustain blooms year-round. Data from Rewalsar Lake highlight how warming-induced eutrophication depletes DO and triggers algal toxins, mirroring Himalayan-wide trends where 60% of lakes face intensified HAB risks (Kumar & Mahajan, 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlBhd2FuIiwiZmFtaWx5IjoiS3VtYXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW1icmlzaCBLdW1hciIsImZhbWlseSI6Ik1haGFqYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyNTIzLTM5NjMiLCIyNTIzLTM5NzEiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMDcvczQyNDUyLTAyMC0zMDgyLTgiLCJtYWciOiIzMDM1OTYwMjI4Iiwib3BlbmFsZXgiOiJXMzAzNTk2MDIyOCJ9LCJ0aXRsZSI6IlRyb3BoaWMgc3RhdHVzIGFuZCBpdHMgcmVndWxhdGluZyBmYWN0b3IgZGV0ZXJtaW5hdGlvbiBhdCB0aGUgUmV3YWxzYXIgTGFrZSwgbm9ydGh3ZXN0IEhpbWFsYXlhIChIUCksIEluZGlhLCBiYXNlZCBvbiBzZWxlY3RlZCBwYXJhbWV0ZXJzIGFuZCBtdWx0aXZhcmlhdGUgc3RhdGlzdGljYWwgYW5hbHlzaXMiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAwNy9zNDI0NTItMDIwLTMwODItOCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9saW5rLnNwcmluZ2VyLmNvbS9jb250ZW50L3BkZi8xMC4xMDA3L3M0MjQ1Mi0wMjAtMzA4Mi04LnBkZiIsImlzc3VlZERhdGUiOiIyMDIwLTA2LTIyIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiU04gQXBwbGllZCBTY2llbmNlcyIsImlzc3VlIjo3LCJ2b2x1bWUiOjIsImltcGFjdEZhY3RvciI6IjcuMzUifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiU3ByaW5nZXIgTmF0dXJlIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo0MCwiY2l0ZWRCeUNvdW50IjoxOX19LCJxdW90ZSI6IkNoYW5naW5nIHF1YWxpdHkgaGFzIGNyZWF0ZWQgdGhlIHdhdGVyIHNjYXJjaXR5IGFuZCAgYSBodWdlIG1hc3Mgb2YgcG9wdWxhdGlvbiB0aHVzIGJlZW4gZGVwcml2ZWQgZnJvbSB1c2luZyAgdGhlIGZyZXNod2F0ZXIgZm9yIHZhcmlvdXMgcHVycG9zZXMgWzMsIDRdLiBIb3dldmVyLCB0aGUgIG9ibm94aW91cyBtYXRlcmlhbHMgdGhhdCBzdGF5IGluIHdhdGVyIGJvZGllcyBmb3IgYSBsb25nLXRpbWUgcGVyaW9kIGJyaW5nIGNoYW5nZXMgaW4gdGhlIHBoeXNpb2NoZW1pY2FsIGNoYXItYWN0ZXJpc3RpY3MgYXMgd2VsbCBhcyB0aGUgbmF0dXJhbCBzY2VuYXJpbyBvZiB0aGUgbGFrZXMgWzVdLiAgVGhlIEhpbWFsYXlhbiBsYWtlcyBhcmUgdW5kZXIgc3RyZXNzIGR1ZSB0byBtaXNtYW5hZ2VkIGh1bWFuIGFjdGl2aXRpZXMsIHdoaWNoIGluY3JlYXNlIG51dHJpZW50cyBhbmQgcG9sbHV0aW9uIGluIHRlcm1zIG9mIGhlYXZ5IG1ldGFscyB0b3hpY2l0eSBhbmQgYmFjdGVyaWFsIGNvbnRhbWluYXRpb24gIFs24oCTOF0uIFRoZSBtYW5hZ2VtZW50IGFuZCBwcmVzZXJ2YXRpb24gb2YgdGhlIGxha2VzIGF0ICBwcmVzZW50IGhhdmUgYmVjb21lIGEgZ3JlYXQgY2hhbGxlbmdlIGZvciBlbnZpcm9ubWVudGFsLWlzdHMgYW5kIG1hbmFnZXJzIFs1LCA5XS4gVGhlIGV1dHJvcGhpY2F0aW9uIGlzIGFsc28gYSBtYWluIHByb2JsZW0gb2YgdGhlICBmcmVzaHdhdGVyIGxha2VzLCBhbmQgaXQgaXMgc3BlY2lmaWVkIGZvciB0aGUgbGFrZXMgaGF2aW5nICBudXRyaWVudCBlbnJpY2htZW50LiBJdCBjYXVzZXMgZXhjZXNzaXZlIGdyb3d0aCBvZiBhbGdhZSAgYW5kIGlzIGNvbnNpZGVyZWQgdG8gYmUgcmVzcG9uc2libGUgZm9yIGRlZ3JhZGluZyB0aGUgIHF1YWxpdGF0aXZlIG5hdHVyZSBvZiB0aGUgbGFrZXMgWzUsIDEwXS4gVGhlIGV1dHJvcGhpY2F0aW9uICBpcyBmb2xsb3dlZCBieSB0aGUgYWxnYWwgYmxvb20sIHdoaWNoIGdlbmVyYXRlcyB0b3hpbiwgIHdvcnNlbnMgd2F0ZXIgcXVhbGl0eSwgZGVwbGV0ZXMgZGlzc29sdmVkIG94eWdlbiAoRE8pICogUGF3YW4gS3VtYXIsIHBhd2FuLmV2c0BnbWFpbC5jb20gfCAxRGVwYXJ0bWVudCBvZiBFbnZpcm9ubWVudGFsIFNjaWVuY2VzLCBDZW50cmFsIFVuaXZlcnNpdHkgb2YgSGltYWNoYWwgUHJhZGVzaCwgRGhhcmFtc2hhbGEgMTc2MjE1LCBJbmRpYS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2OC41Nzg0NjA2OTMzNTkzOCwibGVmdCI6OC41NzE0MTg3NjIyMDcwMzEsIndpZHRoIjo0MC4zMjc1MTQ2NDg0Mzc1LCJoZWlnaHQiOjEuNTE5ODU1MjYwODQ4OTk5fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NzAuMTU4OTk2NTgyMDMxMjUsImxlZnQiOjguNTY5NzM4Mzg4MDYxNTIzLCJ3aWR0aCI6NDAuMzU0NzI0ODg0MDMzMiwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjcxLjczOTU0NzcyOTQ5MjE5LCJsZWZ0Ijo4LjU2OTczODM4ODA2MTUyMywid2lkdGgiOjQwLjM0OTY5MzI5ODMzOTg0NCwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjczLjMyMDA5MTI0NzU1ODYsImxlZnQiOjguNTY4MDU5OTIxMjY0NjQ4LCJ3aWR0aCI6NDAuMDIwMDg4MTk1ODAwNzgsImhlaWdodCI6MS41MTk4NTUyNjA4NDg5OTl9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo3NC45MDA2NDIzOTUwMTk1MywibGVmdCI6OC41NjgwNTk5MjEyNjQ2NDgsIndpZHRoIjozOS45OTk5Mjc1MjA3NTE5NSwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjc2LjQ4MTE4NTkxMzA4NTk0LCJsZWZ0Ijo4LjU2ODA1NzA2MDI0MTcsIndpZHRoIjo0MC4zNTcwNzQ3Mzc1NDg4MywiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjc4LjA2MTcyOTQzMTE1MjM0LCJsZWZ0Ijo4LjU2OTczNjQ4MDcxMjg5LCJ3aWR0aCI6NDAuMzUzNzEzOTg5MjU3ODEsImhlaWdodCI6MS41MTk4NTUyNjA4NDg5OTl9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2MC42NzU3Mjc4NDQyMzgyOCwibGVmdCI6NTEuNDI3MTY1OTg1MTA3NDIsIndpZHRoIjo0MC4zNzg1NjY3NDE5NDMzNiwiaGVpZ2h0IjoxOC45MDU4NTUxNzg4MzMwMDh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo4MS45MjczMzc2NDY0ODQzOCwibGVmdCI6OC40MDE0OTc4NDA4ODEzNDgsIndpZHRoIjo3My45NTQ2OTY2NTUyNzM0NCwiaGVpZ2h0IjoyLjY3ODc2MjY3NDMzMTY2NX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEifX1d. These climate-nutrient synergies manifest as perennial froth in Bengaluru's lakes, where temperatures exceeding 30°C coincide with scum coverage >50% (Nair & Nayak, 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hdnlhIFZpa3JhbWFuIiwiZmFtaWx5IjoiTmFpciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQcmF0ZWVwIEt1bWFyIiwiZmFtaWx5IjoiTmF5YWsifX1dLCJpZHMiOnsiaXNzbiI6WyIyMjk2LTc3NDUiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzODkvZm1hcnMuMjAyMy4xMDg3Mjk2Iiwib3BlbmFsZXgiOiJXNDM4MjcyMzEzMyJ9LCJ0aXRsZSI6IlVuY292ZXJpbmcgd2F0ZXIgcXVhbGl0eSBhbmQgZXZhbHVhdGluZyB2dWxuZXJhYmlsaXRpZXMgb2Ygc21hbGwtc2NhbGUgZmlzaGVyaWVzIGluIENoaWxpa2EgTGFnb29uLCBJbmRpYSIsImFic3RyYWN0IjoiU21hbGwtc2NhbGUgZmlzaGVyaWVzIChTU0ZzKSBwbGF5IGEgc3Ryb25nIHJvbGUgaW4gc3VzdGFpbmluZyBtaWxsaW9ucyBvZiBsaXZlbGlob29kcywgZm9vZCBzZWN1cml0eSwgbnV0cml0aW9uLCBhbmQgaW5jb21lIGdsb2JhbGx5IGJ1dCB0aGUgZmlzaGVycyBlbmdhZ2VkIGluIHRoaXMgc2VjdG9yIHNpbXVsdGFuZW91c2x5IGV4cGVyaWVuY2UgaGlnaCBsZXZlbHMgb2YgdnVsbmVyYWJpbGl0eSBhbmQgcHJvY2Vzc2VzIG9mIG1hcmdpbmFsaXNhdGlvbi4gU2V2ZXJhbCBmYWN0b3JzIGFyZSBhdHRyaWJ1dGVkIHRvIHRoZSBtdWx0aWRpbWVuc2lvbmFsIHZ1bG5lcmFiaWxpdGllcyB0aGUgc21hbGwtc2NhbGUgZmlzaGVycyBleHBlcmllbmNlLCBzcGFubmluZyBib3RoIG5hdHVyYWwgKGUuZy4sIG5hdHVyYWwgZGlzYXN0ZXJzLCBlY29zeXN0ZW0gY2hhbmdlKSBhbmQgYW50aHJvcG9nZW5pYyAoZS5nLiwgcG9saWN5IGNoYW5nZSwgaHlkcm9sb2dpY2FsIGludGVydmVudGlvbnMsIGFxdWFjdWx0dXJlKSBwcmVzc3VyZXMuIFdoaWxlIHRoZXJlIGlzIG11Y2ggbGl0ZXJhdHVyZSBvbiB2YXJpb3VzIG5hdHVyYWwgYW5kIGh1bWFuIGRyaXZlcnMgb2YgdnVsbmVyYWJpbGl0eSBpbiBzbWFsbC1zY2FsZSBmaXNoZXJ5IGNvbW11bml0aWVzLCBhbiBhYnNlbmNlIG9mIHJlc2VhcmNoIGNvbm5lY3RpbmcgdnVsbmVyYWJpbGl0eSB3aXRoIHdhdGVyIHF1YWxpdHkgaXMgZXZpZGVudC4gRmlzaGVyIGNvbW11bml0aWVzIG9mdGVuIHRhbGsgYWJvdXQgZmlzaCBpbiByZWxhdGlvbiB0byB0aGUgaGVhbHRoIG9mIHRoZWlyIGFxdWF0aWMgaGFiaXRhdHMgd2hlcmVpbiB3YXRlciBxdWFsaXR5IGlzIHNlZW4gYXMgYSBrZXkgcGFyYW1ldGVyLiBUaGUgbGluayBiZXR3ZWVuIGhlYWx0aHkgZmlzaCBhbmQgZ29vZCBxdWFsaXR5IHdhdGVyIGhhcyBzaWduaWZpY2FudCBpbXBsaWNhdGlvbnMgZm9yIHN0cm9uZyBhbmQgdmlhYmxlIGZpc2hpbmcgY29tbXVuaXRpZXMuIFRoaXMgcGFwZXIgZXhhbWluZXMgdGhlc2UgbGlua3MgZnVydGhlciBieSBmb2N1c2luZyBvbiB0aGUgbmF0dXJlIG9mIHZ1bG5lcmFiaWxpdGllcyBjYXVzZWQgYnkgd2F0ZXIgcXVhbGl0eSBjaGFuZ2VzIGluIHRoZSBzbWFsbC1zY2FsZSBmaXNoZXJ5IHN5c3RlbSBvZiBDaGlsaWthIExhZ29vbiBpbiBJbmRpYS4gV2UgdW5kZXJ0YWtlIGRldGFpbGVkIGFuYWx5c2lzIG9mIHRoZSBpbnZhc2l2ZSBzaHJpbXAgYXF1YWN1bHR1cmUgYWN0aXZpdGllcyBhbmQgaHlkcm9sb2dpY2FsIGludGVydmVudGlvbnMgZm9yIG9wZW5pbmcgb2YgYSBsYWdvb24gaW5sZXQgd2l0aCB0aGUgQmF5IG9mIEJlbmdhbCBhcyB0d28gZG9taW5hbnQgZHJpdmVycyBhZHZlcnNlbHkgaW1wYWN0aW5nIHdhdGVyIHF1YWxpdHkgYW5kIGluY3JlYXNpbmcgdnVsbmVyYWJpbGl0aWVzIG9mIHRoZSBlbnRpcmUgc21hbGwtc2NhbGUgZmlzaGVyaWVzIHNvY2lhbC1lY29sb2dpY2FsIHN5c3RlbS4gT3VyIGFuYWx5c2lzIHN1Z2dlc3RzIHRoYXQgdGhlcmUgYXJlIHN0cm9uZyBpbnRlcmNvbm5lY3Rpb25zIGJldHdlZW4gY2hhbmdlcyBpbiB3YXRlciBxdWFsaXR5IGFuZCB0aGUgbGV2ZWxzIG9mIHZ1bG5lcmFiaWxpdGllcyBpbiB0aGUgU1NGcyBvZiBDaGlsaWthIExhZ29vbi4gUG9sbHV0YW50cyBzdWNoIGFzIHBlc3RpY2lkZXMsIGFuZCBvcmdhbmljIGNvbXBvdW5kcyBhY2N1bXVsYXRlIGluIGZpc2ggdGlzc3VlcyBhbmQgYWZmZWN0IHRoZWlyIGdyb3d0aCwgcmVwcm9kdWN0aW9uLCBhbmQgb3ZlcmFsbCBoZWFsdGguIFRoaXMgbGVkIHRvIGRlY2xpbmVzIGluIGZpc2ggcG9wdWxhdGlvbnMsIG1ha2luZyBpdCBtb3JlIGRpZmZpY3VsdCBmb3IgZmlzaGVycyB0byBtYWtlIGEgbGl2aW5nLiBJbiBhZGRpdGlvbiB0byBkaXJlY3QgaW1wYWN0cyBvbiBmaXNoIHBvcHVsYXRpb25zLCBwb29yIHdhdGVyIHF1YWxpdHkgYWxzbyBoYXMgaW5kaXJlY3QgZWZmZWN0cyBvbiB0aGUgc29jaWFsIGFuZCBlY29ub21pYyB2dWxuZXJhYmlsaXR5IG9mIFNTRnMuIEZvciBleGFtcGxlLCBjb250YW1pbmF0aW9uIG9mIHdhdGVyIHNvdXJjZXMgbGVkIHRvIHRoZSByZWR1Y2VkIG51bWJlciBvZiBmaXNoIHNwZWNpZXMgcmVkdWNpbmcgdGhlIGFtb3VudCBvZiB0aW1lIGZpc2hlcnMgY2FuIHNwZW5kIG9uIGZpc2hpbmcgYWN0aXZpdGllcy4gVGhpcyBhbHNvIGFmZmVjdGVkIHRoZSBtYXJrZXRhYmlsaXR5IG9mIGZpc2ggcHJvZHVjdHMsIHJlZHVjZWQgaW5jb21lIGFuZCBpbmNyZWFzZWQgcG92ZXJ0eS4gVG8gZnVsbHkgdW5kZXJzdGFuZCB0aGUgaW50ZXJjb25uZWN0aW9ucyBiZXR3ZWVuIHdhdGVyIHF1YWxpdHkgYW5kIHZ1bG5lcmFiaWxpdHkgaW4gU1NGcyBpbiBDaGlsaWthIExhZ29vbiwgaXQgaXMgaW1wb3J0YW50IHRvIGNvbnNpZGVyIGJvdGggZW52aXJvbm1lbnRhbCBhbmQgc29jaWFsIGZhY3RvcnMsIGFzIHdlbGwgYXMgdGhlIGNvbXBsZXggZmVlZGJhY2sgbG9vcHMgYmV0d2VlbiB0aGVzZSBmYWN0b3JzLiBUaGUgc3R1ZHkgaGVscHMgaW4gYnJpZGdpbmcgYSBjcnVjaWFsIGdhcCBpbiBvdXIgdW5kZXJzdGFuZGluZyBvZiB0aGUgcm9sZSBvZiB3YXRlciBxdWFsaXR5IGluIHZ1bG5lcmFiaWxpdHkgYW5hbHlzaXMgd2l0aGluIHJlc291cmNlIGRlcGVuZGVudCBjb21tdW5pdGllcy4gV2UgY29uY2x1ZGUgd2l0aCBrZXkgaW5zaWdodHMgb24gcG9zc2libGUgY29waW5nIHJlc3BvbnNlcyBhbmQgYWRhcHRpdmUgY2FwYWNpdHkgbmVjZXNzYXJ5IGZvciB0aGUgc21hbGwtc2NhbGUgZmlzaGVyaWVzIGNvbW11bml0aWVzIHRvIHRyYW5zaXRpb24gdG93YXJkIHZpYWJpbGl0eS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMzM4OS9mbWFycy4yMDIzLjEwODcyOTYiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vd3d3LmZyb250aWVyc2luLm9yZy9hcnRpY2xlcy8xMC4zMzg5L2ZtYXJzLjIwMjMuMTA4NzI5Ni9wZGYiLCJpc3N1ZWREYXRlIjoiMjAyMy0wNi0yOSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZyb250aWVycyBpbiBNYXJpbmUgU2NpZW5jZSIsInZvbHVtZSI6MTAsImltcGFjdEZhY3RvciI6IjQuMjgifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRnJvbnRpZXJzIE1lZGlhIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo1MSwiY2l0ZWRCeUNvdW50Ijo1fX0sInF1b3RlIjoiV2F0ZXIgcXVhbGl0eSBvZiBDaGlsaWthIExhZ29vbiBoYXMgZGVncmFkZWQgZHVlIHRvIGFudGhyb3BvZ2VuaWMgaW50ZXJ2ZW50aW9ucyBhbmQgbmF0dXJhbCBmb3JjZXMgb2YgY2hhbmdlLiBWYXJpb3VzIGFjdGl2aXRpZXMgaW4gdGhlIENoaWxpa2EgTGFnb29uLCBzdWNoIGFzIGluZHVzdHJpYWwgd2FzdGV3YXRlciBkaXNwb3NhbCwgc2V3YWdlIGR1bXBpbmcsIGFxdWFjdWx0dXJlLCBoeWRyb2xvZ2ljYWwgaW50ZXJ2ZW50aW9ucywgYW5kIGN5Y2xvbmVzLCBhcmUgcHJvZHVjaW5nIHNhbGluaXR5IGNoYW5nZXMsIHNlZGltZW50IGRlcG9zaXRpb24sIG51dHJpZW50IGVucmljaG1lbnQsIGV1dHJvcGhpY2F0aW9uLCBhbmQgZGVhZCB6b25lcywgYXMgc3RhdGVkLiBGaXNoIHN1cnZpdmFsIGlzIGF0IHJpc2sgYXMgYSByZXN1bHQgb2YgdGhlIHNpZ25pZmljYW50IHJlZHVjdGlvbiBpbiBlbnZpcm9ubWVudGFsIGNpcmN1bXN0YW5jZXMsIHdoaWNoIGRpbWluaXNoZXMgZmlzaGVybWVu4oCZcyBpbmNvbWUgYW5kIHB1c2hlcyB0aGVtIG91dCBvZiBzb2NpZXR5LiBUaGVzZSBjYXVzZXMgYWZmZWN0aW5nIHRoZSB3YXRlciBxdWFsaXR5IG9mIHRoZSBMYWdvb24gZGVwaWN0IG1hbnkgZmFjZXMgb2YgdnVsbmVyYWJpbGl0eSBpbiBTU0YgY29tbXVuaXRpZXMuIEFzIHRoZSBDaGlsaWthIExhZ29vbiBlbnZpcm9ubWVudCBpcyBleHBlY3RlZCB0byB1bmRlcmdvIGluY3JlYXNpbmdseSBkcmFzdGljIGNoYW5nZXMgKHN1Y2ggYXMgY2F0Y2ggY29tcG9zaXRpb24sIGNhdGNoIGNhcGFjaXR5LCBiaW9kaXZlcnNpdHkgdmFyaWF0aW9ucywgd2F0ZXIgcXVhbGl0eSwgYW5kIGZpc2hlcnkgcmV2ZW51ZXMpLCBpbnRlcm5hdGlvbmFsIHBvbGljeSBoYXMgYmVjb21lIG1vcmUgaW50ZXJlc3RlZCBpbiBhZGFwdGF0aW9uIG1lYXN1cmVzLiBHaXZlbiBob3cgZGl2ZXJzZSBTU0YgZWNvc3lzdGVtcyBhcmUgZXhwZWN0ZWQgdG8gYmUgYmV0d2VlbiBjb250aW5lbnRzLCB0aGVyZSBpcyBubyDigJxvbmUgc2l6ZSBmaXRzIGFsbOKAnSBhcHByb2FjaC4gVGhlIHBhcGVyIG91dGxpbmVkIHNldmVyYWwgY29waW5nIGFuZCBhZGFwdGF0aW9uIG1lY2hhbmlzbXMgdG8gaW1wcm92ZSB3YXRlciBxdWFsaXR5IGFuZCBjb250aW51ZSBmaXNoaW5nIGluIGEgc3VzdGFpbmFibGUgd2F5LiBBZGFwdGF0aW9uIHN0cmF0ZWdpZXMgY2FuIG9jY2FzaW9uYWxseSBoYXZlIHVuZm9yZXNlZW4gY29uc2VxdWVuY2VzIGZvciBmaXNoZXJpZXMgYW5kIGZpc2hpbmcgY29tbXVuaXRpZXMuIEFueSB1bmV4cGVjdGVkIGNvbnNlcXVlbmNlcyBvZiBhZGFwdGF0aW9uIGNhbiBiZSBtaXRpZ2F0ZWQgdGhyb3VnaCBsb25nLXRlcm0gcGxhbm5pbmcgYW5kIGlkZW50aWZpY2F0aW9uIG9mIGZ1dHVyZSBvcHRpb25zIHRocm91Z2ggc2NlbmFyaW8gYW5hbHlzaXMuIFRoZSBmaW5kaW5ncyBvZiB0aGlzIHBhcGVyIGNhbiBiZSB1dGlsaXplZCB0byBkZXZlbG9wIHBvbGljaWVzIHRvIHByb3RlY3Qgd2F0ZXIgcXVhbGl0eSBhbmQgU1NGcyBpbiBhIHZhcmlldHkgb2YgY29hc3RhbCBsYWdvb24gc2V0dGluZ3MuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjIyLjI4NTAzNjA4NzAzNjEzMywibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ5MzA1MzQzNjI3OTMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjIzLjc3MzUxOTUxNTk5MTIxLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDgzMzY0MTA1MjI0NjEsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjI1LjI1NTU4NDcxNjc5Njg3NSwibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ4OTU1OTE3MzU4Mzk4NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjYuNzQ0MDY4MTQ1NzUxOTUzLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNTA2ODA1NDE5OTIxODc1LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyOC4yMjYxMzMzNDY1NTc2MTcsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40OTA0NjMyNTY4MzU5NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjkuNzE0NjE2Nzc1NTEyNjk1LCJsZWZ0Ijo4LjMzMTU5MjU1OTgxNDQ1Mywid2lkdGgiOjQwLjQ5NDA5ODY2MzMzMDA4LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozMS4xOTY2ODM4ODM2NjY5OTIsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40OTQzOTIzOTUwMTk1MywiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MzIuNjg1MTY1NDA1MjczNDQsImxlZnQiOjguMzMxNTk2Mzc0NTExNzE5LCJ3aWR0aCI6NDAuNDg3NDQ1ODMxMjk4ODMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM0LjE2NzIzMjUxMzQyNzczNCwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40OTc3NDU1MTM5MTYwMTYsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM1LjY1NTcxNTk0MjM4MjgxLCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ5ODYzODE1MzA3NjE3LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozNy4xMzc3ODMwNTA1MzcxMSwibGVmdCI6MTAuNjg1MTg2Mzg2MTA4Mzk4LCJ3aWR0aCI6MzguMTQxMTA1NjUxODU1NDcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM4LjYyNjI2NjQ3OTQ5MjE5LCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4NzE1MjA5OTYwOTM3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDAuMTA4MzMzNTg3NjQ2NDg0LCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4OTg1NjcxOTk3MDcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQxLjU5NjgxMzIwMTkwNDMsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDg4NjY2NTM0NDIzODMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQzLjA3ODg4MDMxMDA1ODU5NCwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40ODE4NjExMTQ1MDE5NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDQuNTY3MzYzNzM5MDEzNjcsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDgzMDU4OTI5NDQzMzYsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQ2LjA0OTQzMDg0NzE2Nzk3LCJsZWZ0Ijo4LjMzMzEwODkwMTk3NzUzOSwid2lkdGgiOjQwLjUwMDc0NzY4MDY2NDA2LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo0Ny41Mzc5MTQyNzYxMjMwNSwibGVmdCI6OC4zMzMxMDg5MDE5Nzc1MzksIndpZHRoIjo0MC40OTU2MDU0Njg3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDkuMDE5OTgxMzg0Mjc3MzQ0LCJsZWZ0Ijo4LjMzMzEwNjA0MDk1NDU5LCJ3aWR0aCI6NDAuNTA2ODE2ODY0MDEzNjcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUwLjUwODQ2NDgxMzIzMjQyLCJsZWZ0Ijo4LjMzMzEwNjA0MDk1NDU5LCJ3aWR0aCI6NDAuNDg4NTA2MzE3MTM4NjcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUxLjk5MDUyODEwNjY4OTQ1LCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDk3NzI2NDQwNDI5NjksImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUzLjQ3OTAxMTUzNTY0NDUzLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6MzMuMTU5NDQ2NzE2MzA4NTk0LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo1NC45NjEwNzg2NDM3OTg4MywibGVmdCI6MTAuNjgzNjcwMDQzOTQ1MzEyLCJ3aWR0aCI6MzguMTM2NTk2Njc5Njg3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NTYuNDQ5NTYyMDcyNzUzOTA2LCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDg3MTQ0NDcwMjE0ODQ0LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEzIn19XQ%3D%3D.

Projections indicate that under RCP 4.5 scenarios, Indian lake temperatures could rise 2-4°C by 2050, potentially rendering 70-80% of systems chronically eutrophic unless nutrient controls intensify (Jilbert et al., 2020; Nõges et al., 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRvbSIsImZhbWlseSI6IkppbGJlcnQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUmFvdWzigJBNYXJpZSIsImZhbWlseSI6IkNvdXR1cmUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQnJpYW4iLCJmYW1pbHkiOiJIdXNlciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLYWxldmkiLCJmYW1pbHkiOiJTYWxvbmVuIn19XSwiaWRzIjp7Imlzc24iOlsiMDAxOC04MTU4IiwiMTU3My01MTE3Il0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDA3L3MxMDc1MC0wMjAtMDQ0NTcteCIsIm1hZyI6IjMxMDA0NTkwNTAiLCJvcGVuYWxleCI6IlczMTAwNDU5MDUwIn0sInRpdGxlIjoiUHJlZmFjZTogUmVzdG9yYXRpb24gb2YgZXV0cm9waGljIGxha2VzOiBjdXJyZW50IHByYWN0aWNlcyBhbmQgZnV0dXJlIGNoYWxsZW5nZXMiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAwNy9zMTA3NTAtMDIwLTA0NDU3LXgiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vbGluay5zcHJpbmdlci5jb20vY29udGVudC9wZGYvMTAuMTAwNy9zMTA3NTAtMDIwLTA0NDU3LXgucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjAtMTEtMTYiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA3IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJIeWRyb2Jpb2xvZ2lhIiwiaXNzdWUiOjIxLCJ2b2x1bWUiOjg0NywiaW1wYWN0RmFjdG9yIjoiMy4wMCJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjQzNDMsImxhc3RQYWdlIjo0MzU3fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlNwcmluZ2VyIFNjaWVuY2UrQnVzaW5lc3MgTWVkaWEifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjk1LCJjaXRlZEJ5Q291bnQiOjU4fX0sInF1b3RlIjoiU2ltaWxhciBkZWdyYWRhdGlvbiBvZiBmcmVzaHdhdGVyIGVjb3N5c3RlbXMgaXMgb2JzZXJ2ZWQgd29ybGR3aWRlLCB3aXRoIHRoZSB0aW1lbGluZSBvZiBodW1hbiBwcmVzc3VyZSBkaWN0YXRlZCBieSB0cmVuZHMgaW4gdXJiYW5pemF0aW9uIGFuZCBpbnRlbnNpdmUgYWdyaWN1bHR1cmUgaW4gZGlmZmVyZW50IGdlb2dyYXBoaWMgcmVnaW9ucyAoZS5nLiwgVmVubmVtbyBldCBhbC4sIDIwMDkpLiBUaGUgcHJpbmNpcGFsIGRyaXZlciBvZiBlY29zeXN0ZW0gc3RyZXNzIGluIG1vc3QgYXF1YXRpYyBzeXN0ZW1zIGlzIGV1dHJvcGhpY2F0aW9uLCB0aGUgZXhjZXNzIGdyb3d0aCBvZiBhbGdhZSBkcml2ZW4gYnkgYW50aHJvcG9nZW5pYyBpbnB1dHMgb2YgdGhlIG1hY3JvbnV0cmllbnRzIG5pdHJvZ2VuIChOKSBhbmQgcGhvc3Bob3J1cyAoUCkgKFNtaXRoLCAyMDAzOyBCaXJrIGV0IGFsLiwgMjAyMCkuIE51dHJpZW50cyBhcmUgZW1pdHRlZCBmcm9tIGJvdGggcG9pbnQgYW5kIGRpZmZ1c2Ugc291cmNlcyBpbiB0aGUgaHVtYW4gZW52aXJvbm1lbnQgYW5kIGVudGVyIGFxdWF0aWMgc3lzdGVtcyB2aWEgc3VyZmFjZSBydW5vZmYsIGFzIHdlbGwgYXMgZ3JvdW5kLXdhdGVyIGFuZCBhdG1vc3BoZXJpYyBkZXBvc2l0aW9uIChQYWVybCwgMTk5NykuIEV1dHJvcGhpY2F0aW9uIG9mdGVuIGxlYWRzIHRvIHRoZSBkb21pbmFuY2Ugb2YgcHJpbWFyeSBwcm9kdWN0aW9uIGJ5IGN5YW5vYmFjdGVyaWEsIGluY2x1ZGluZyB0b3hpYyBnZW5lcmEgc3VjaCBhcyBEb2xpY2hvc3Blcm11bSwgQXBoYW5pem9tZW5vbiwgYW5kIE1pY3JvY3lzdGlzIChIdWlzbWFuIGV0IGFsLiwgMjAxOCksIGFzIHdlbGwgYXMgb3h5Z2VuIGRlcGxldGlvbiBhbmQgaGFiaXRhdCBkZWdyYWRhdGlvbiBpbiBsYWtlcyAoQXJlbmQgZXQgYWwuLCAyMDExKS4gQmVjYXVzZSBsYWtlcyBwcm92aWRlIGEgcmFuZ2Ugb2YgY3J1Y2lhbCBlY29zeXN0ZW0gc2VydmljZXMgKFdpbHNvbiAmIENhcnBlbnRlciwgMTk5OSksIHRoZXJlIGlzIGEgZ3Jvd2luZyBuZWVkIGZvciBtYW5hZ2VtZW50IGFwcHJvYWNoZXMgdGhhdCByZXN0b3JlIHdhdGVyIHF1YWxpdHkgaW4gZnJlc2h3YXRlciBzeXN0ZW1zLiBXaXRoaW4gdGhlIGZyYW1ld29yayBvZiBVbml0ZWQgTmF0aW9ucyBTdXN0YWluYWJsZSBEZXZlbG9wLW1lbnQgR29hbCA2IChDbGVhbiBXYXRlciBhbmQgU2FuaXRhdGlvbiksIHNldmVyYWwgZGlzdGluY3QgdGFyZ2V0cyByZWxhdGUgdG8gdGhlIGltcHJvdmVtZW50IG9mIHdhdGVyIHF1YWxpdHkgYW5kIHByb3RlY3Rpb24gb2YgYXF1YXRpYyBlY29zeXN0ZW1zIGJ5IDIwMzAgKFVuaXRlZCBOYXRpb25zLCAyMDE1KS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyOC4yOTg4NjQzNjQ2MjQwMjMsImxlZnQiOjguNTQ5MjA1NzgwMDI5Mjk3LCJ3aWR0aCI6MzkuMzcxNDk0MjkzMjEyODksImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MjkuOTkxMTM0NjQzNTU0Njg4LCJsZWZ0Ijo4LjU0OTIwNTc4MDAyOTI5Nywid2lkdGgiOjM5LjM2NTExOTkzNDA4MjAzLCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMxLjY4MzQwNjgyOTgzMzk4NCwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNjk2NjMyMzg1MjUzOSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozMy4zNjc5NzMzMjc2MzY3MiwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNzU2NzkwMTYxMTMyOCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNS4wNjAyNDkzMjg2MTMyOCwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNzA5NjQwNTAyOTI5NywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNi43NTI1MTc3MDAxOTUzMSwibGVmdCI6OC41NDkyMDg2NDEwNTIyNDYsIndpZHRoIjozOS4zNzY5NjA3NTQzOTQ1MywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC40MzcwODgwMTI2OTUzMSwibGVmdCI6OC41NDkyMDg2NDEwNTIyNDYsIndpZHRoIjozOS4zNjY1Njk1MTkwNDI5NywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0MC4xMjkzNTYzODQyNzczNDQsImxlZnQiOjguNTQ5MjA4NjQxMDUyMjQ2LCJ3aWR0aCI6MzkuMzY0Mzc5ODgyODEyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0MS44MjE2MjQ3NTU4NTkzNzUsImxlZnQiOjguNTQ5MjA4NjQxMDUyMjQ2LCJ3aWR0aCI6MzkuMzY1NjczMDY1MTg1NTUsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDMuNTA2MTk1MDY4MzU5Mzc1LCJsZWZ0Ijo4LjU0OTM4OTgzOTE3MjM2Mywid2lkdGgiOjM5LjM3MDM5MTg0NTcwMzEyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0NS4xOTg0NjM0Mzk5NDE0MDYsImxlZnQiOjguNTQ5NTcyOTQ0NjQxMTEzLCJ3aWR0aCI6MzkuMzY1ODQ0NzI2NTYyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ni44OTA3MzU2MjYyMjA3LCJsZWZ0Ijo4LjU0OTU3Mjk0NDY0MTExMywid2lkdGgiOjM5LjM3Mjk1NTMyMjI2NTYyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0OC41NzUzMDIxMjQwMjM0NCwibGVmdCI6OC41NDk1NzI5NDQ2NDExMTMsIndpZHRoIjozOS4zNzI0MjEyNjQ2NDg0NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1MC4yNjc1NzgxMjUsImxlZnQiOjguNTQ5NTc2NzU5MzM4Mzc5LCJ3aWR0aCI6MzkuMzcxMTI0MjY3NTc4MTI1LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjUxLjk1OTg0MjY4MTg4NDc2NiwibGVmdCI6OC41NDk1NzY3NTkzMzgzNzksIndpZHRoIjozOS4zNjc4MzU5OTg1MzUxNTYsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTMuNTc0MTE5NTY3ODcxMDk0LCJsZWZ0Ijo4LjU0OTU3Njc1OTMzODM3OSwid2lkdGgiOjM5LjM3MDc3NzEzMDEyNjk1LCJoZWlnaHQiOjEuMzM2ODk1NTg1MDYwMTE5Nn19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjU1LjI2NjM5MTc1NDE1MDM5LCJsZWZ0Ijo4LjU0OTc2MDgxODQ4MTQ0NSwid2lkdGgiOjM5LjM2NjU4MDk2MzEzNDc2NiwiaGVpZ2h0IjoxLjMzNjg5NTU4NTA2MDExOTZ9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1Ny4wMjg5NTM1NTIyNDYwOTQsImxlZnQiOjguNTQ5OTQ0ODc3NjI0NTEyLCJ3aWR0aCI6MzkuMzYyMDI2MjE0NTk5NjEsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTguNzEzNTIzODY0NzQ2MDk0LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjkuMjk5OTgyMDcwOTIyODUyLCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjYwLjQwNTc5MjIzNjMyODEyNSwibGVmdCI6MTAuNjIyNDU1NTk2OTIzODI4LCJ3aWR0aCI6MzcuMjg3ODY4NDk5NzU1ODYsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NjIuMDk4MDYwNjA3OTEwMTU2LCJsZWZ0Ijo4LjU0OTk0MzkyMzk1MDE5NSwid2lkdGgiOjM5LjM3MzMyNTM0NzkwMDM5LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjYzLjc4MjYzMDkyMDQxMDE1NiwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzMxMjY5ODM2NDI1OCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2NS40NzQ5MDY5MjEzODY3MiwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNjUxMTk5MzQwODIwMywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2Ny4xNjcxNzUyOTI5Njg3NSwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNjk0ODAxMzMwNTY2NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2OC43ODE0NDgzNjQyNTc4MSwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzczMzg0MDk0MjM4MywiaGVpZ2h0IjoxLjMzNjg5MTUzMTk0NDI3NX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjcwLjU0NDAxMzk3NzA1MDc4LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjM5LjM2OTMwODQ3MTY3OTY5LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjcyLjIzNjI4OTk3ODAyNzM0LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjM5LjM3MjAzMjE2NTUyNzM0NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo3My45MjA4NDUwMzE3MzgyOCwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzEzMDczNzMwNDY4NzUsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNDM0NCJ9fSx7Im9yaWdpbiI6NSwid29yayI6eyJ0eXBlIjo4LCJzdWJ0eXBlIjp7Im9uZW9mS2luZCI6ImFydGljbGVUeXBlIiwiYXJ0aWNsZVR5cGUiOjF9LCJjb250cmlidXRvcnMiOlt7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJUaWluYSIsImZhbWlseSI6Ik7DtWdlcyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJOYXNpbWUiLCJmYW1pbHkiOiJKYW5hdGlhbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJSZWV0IiwiZmFtaWx5IjoiTGF1Z2FzdGUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUGVldGVyIiwiZmFtaWx5IjoiTsO1Z2VzIn19XSwiaWRzIjp7Imlzc24iOlsiMjM1MS05ODk0Il0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDE2L2ouZ2VjY28uMjAyMC5lMDEzNjkiLCJtYWciOiIzMTA1NDQ2MDkwIiwib3BlbmFsZXgiOiJXMzEwNTQ0NjA5MCIsImV4YSI6Imh0dHBzOi8vd3d3LnNjaWVuY2VkaXJlY3QuY29tL3NjaWVuY2UvYXJ0aWNsZS9waWkvUzIzNTE5ODk0MjAzMDkxMDAifSwidGl0bGUiOiJQb3N0LXNvdmlldCBjaGFuZ2VzIGluIG5pdHJvZ2VuIGFuZCBwaG9zcGhvcnVzIHN0b2ljaGlvbWV0cnkgaW4gdHdvIGxhcmdlIG5vbi1zdHJhdGlmaWVkIGxha2VzIGFuZCB0aGUgaW1wYWN0IG9uIHBoeXRvcGxhbmt0b24iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAxNi9qLmdlY2NvLjIwMjAuZTAxMzY5IiwicmVzb3VyY2VVcmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAxNi9qLmdlY2NvLjIwMjAuZTAxMzY5IiwiaXNzdWVkRGF0ZSI6IjIwMjAtMTEtMjAiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTExIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJHbG9iYWwgRWNvbG9neSBhbmQgQ29uc2VydmF0aW9uIiwidm9sdW1lIjoyNCwiaW1wYWN0RmFjdG9yIjoiMy41OSJ9LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJFbHNldmllciBCViJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NzEsImNpdGVkQnlDb3VudCI6MTR9fSwicXVvdGUiOiJSYW5kb20gRm9yZXN0IGFuYWx5c2lzIGluZGljYXRlZCBhbiBpbXBvcnRhbnQgcm9sZSBvZiBsaWdodCBsaW1pdGF0aW9uIGluIGJvdGggbGFrZXMuIENvbnN0YW50bHkgbG93ZXIgbGV2ZWxzIG9mIE4gaW4gdGhlIGRlZXBlciBMYWtlIFBlaXBzaSBmYXZvdXJlZCBOMi1maXhpbmcgc3BlY2llcywgd2hpY2gsIGFzIGEgcGFyYWRveCwgYmVjYW1lIFAtbGltaXRlZC4gQXMgY2xpbWF0ZSB3YXJtaW5nIHJlaW5mb3JjZXMgZXV0cm9waGljYXRpb24gcGhlbm9tZW5hIGluIGxha2VzIGJ5IGluY3JlYXNpbmcgaW50ZXJuYWwgbnV0cmllbnQgbG9hZGluZyBhbmQgZmF2b3VyaW5nIGJsb29tLWZvcm1pbmcgY3lhbm9iYWN0ZXJpYSwgbW9yZSBzdHJpbmdlbnQgbWVhc3VyZXMgd291bGQgYmUgbmVlZGVkIHRvIGZ1cnRoZXIgbGltaXQgbnV0cmllbnQgbG9hZHMgKGVzcGVjaWFsbHkgdGhhdCBvZiBQKSB0byBsYWtlcyB0aHJvdWdoIGltcHJvdmVkIHdhc3Rld2F0ZXIgdHJlYXRtZW50IGFuZCBpbmNyZWFzZWQgZWZmaWNpZW5jeSBvZiBmZXJ0aWxpc2VyIGFwcGxpY2F0aW9uLiBMb25nLXRlcm0gY2hhbmdlcyBpbiBudXRyaWVudCBsb2FkaW5nIGFuZCBzdG9pY2hpb21ldHJ5IGluIHdhdGVyIGJvZGllcyByZWZsZWN0IGNoYW5nZXMgaW4gYWdyaWN1bHR1cmFsIHByYWN0aWNlcyBhbmQgd2FzdGV3YXRlciB0cmVhdG1lbnQuIn1d. Feedbacks like permafrost thaw in northern lakes or glacial melt adding silica imbalances further favor toxin-producers (Haq et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjQsInN1YnR5cGUiOnt9LCJjb250cmlidXRvcnMiOlt7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJadWxmcWFydWwiLCJmYW1pbHkiOiJIYXEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU25vYmVyIiwiZmFtaWx5IjoiSXJzaGFkIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkF6bWF0IEFsYW0iLCJmYW1pbHkiOiJLaGFuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNoZWlraCBGLiIsImZhbWlseSI6IkFobWFkIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNob3drZWVuIE11emFtaWwiLCJmYW1pbHkiOiJCYXNoaXIifX1dLCJpZHMiOnsiaXNzbiI6WyIyNjMyLTA1MTciXSwiaXNibiI6W10sImRvaSI6IjEwLjU3NzIvaW50ZWNob3Blbi4xMDA2NTc4Iiwib3BlbmFsZXgiOiJXNDQwNDUyOTIwMyJ9LCJ0aXRsZSI6IkFkdmFuY2VzIGluIE1hbmFnaW5nIE5pdHJvZ2VuIGFuZCBQaG9zcGhvcnVzIEVtaXNzaW9ucyBpbiBSdW1pbmFudHM6IEEgSG9saXN0aWMgQXBwcm9hY2giLCJhYnN0cmFjdCI6IuKAnEJhbGFuY2luZyBOdXRyaXRpb24gYW5kIEVudmlyb25tZW50YWwgU3VzdGFpbmFiaWxpdHk6IEFkdmFuY2VzIGluIE1hbmFnaW5nIE5pdHJvZ2VuIGFuZCBQaG9zcGhvcnVzIEVtaXNzaW9ucyBpbiBSdW1pbmFudHPigJ0gZXhwbG9yZXMgdGhlIGludHJpY2F0ZSByZWxhdGlvbnNoaXAgYmV0d2VlbiBydW1pbmFudCBudXRyaXRpb24gYW5kIGVudmlyb25tZW50YWwgc3Rld2FyZHNoaXAuIEl0IGRlbHZlcyBpbnRvIHRoZSBkeW5hbWljcyBvZiBuaXRyb2dlbiBhbmQgcGhvc3Bob3J1cyBtZXRhYm9saXNtIGluIHJ1bWluYW50cywgYWRkcmVzc2luZyB0aGUgZW52aXJvbm1lbnRhbCByZXBlcmN1c3Npb25zIG9mIHRoZWlyIGVtaXNzaW9ucy4gVGhlc2UgZW1pc3Npb25zLCBwcmltYXJpbHkgaW4gdGhlIGZvcm0gb2YgYW1tb25pYSBhbmQgcGhvc3BoYXRlcywgcG9zZSBzaWduaWZpY2FudCBlbnZpcm9ubWVudGFsIGNoYWxsZW5nZXMsIGNvbnRyaWJ1dGluZyB0byBhaXIgYW5kIHdhdGVyIHBvbGx1dGlvbiwgc29pbCBkZWdyYWRhdGlvbiwgYW5kIGNsaW1hdGUgY2hhbmdlLiBUaGUgY2hhcHRlciBoaWdobGlnaHRzIGlubm92YXRpdmUgc3RyYXRlZ2llcyBhbmQgdGVjaG5vbG9naWNhbCBicmVha3Rocm91Z2hzIGFpbWVkIGF0IG1pdGlnYXRpbmcgbml0cm9nZW4gYW5kIHBob3NwaG9ydXMgZXhjcmV0aW9uLCBjcnVjaWFsIGZvciByZWR1Y2luZyBlbnZpcm9ubWVudGFsIHBvbGx1dGlvbi4gSXQgb2ZmZXJzIGluc2lnaHRzIGludG8gZGlldGFyeSBhZGp1c3RtZW50cywgZmVlZCBhZGRpdGl2ZXMsIGFuZCBiZXN0IHByYWN0aWNlcyBmb3Igc3VzdGFpbmFibGUgcnVtaW5hbnQgcHJvZHVjdGlvbi4gS2V5IGFyZWFzIG9mIGZvY3VzIGluY2x1ZGUgZGlldGFyeSBhZGp1c3RtZW50cyB0YWlsb3JlZCB0byBvcHRpbWl6ZSBudXRyaWVudCB1dGlsaXphdGlvbiBhbmQgcmVkdWNlIHdhc3RlLCB0aGUgdXRpbGl6YXRpb24gb2YgZmVlZCBhZGRpdGl2ZXMgdG8gZW5oYW5jZSBudXRyaWVudCBhYnNvcnB0aW9uIGFuZCBtaW5pbWl6ZSBleGNyZXRpb24sIGFuZCB0aGUgaW1wbGVtZW50YXRpb24gb2YgYmVzdCBtYW5hZ2VtZW50IHByYWN0aWNlcyB0byBwcm9tb3RlIHN1c3RhaW5hYmxlIHJ1bWluYW50IHByb2R1Y3Rpb24uIEJ5IGludGVncmF0aW5nIHNjaWVudGlmaWMgZmluZGluZ3Mgd2l0aCBwcmFjdGljYWwgYXBwbGljYXRpb25zLCB0aGlzIHJldmlldyB1bmRlcnNjb3JlcyB0aGUgaW1wb3J0YW5jZSBvZiBhY2hpZXZpbmcgaGFybW9ueSBiZXR3ZWVuIG51dHJpdGlvbiBvcHRpbWl6YXRpb24gYW5kIGVudmlyb25tZW50YWwgcmVzcG9uc2liaWxpdHkgaW4gbW9kZXJuIGFncmljdWx0dXJlLiBUaHJvdWdoIHRoaXMgY29tcHJlaGVuc2l2ZSBhbmFseXNpcywgdGhlIGNoYXB0ZXIgYWltcyBmb3IgYmFsYW5jaW5nIHRoZSBkdWFsIGdvYWxzIG9mIG9wdGltaXppbmcgcnVtaW5hbnQgbnV0cml0aW9uIGFuZCBtaW5pbWl6aW5nIGVudmlyb25tZW50YWwgaW1wYWN0LCB0aGVyZWJ5IGNvbnRyaWJ1dGluZyB0byBtb3JlIHN1c3RhaW5hYmxlIGFuZCByZXNpbGllbnQgYWdyaWN1bHR1cmFsIHN5c3RlbXMuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjU3NzIvaW50ZWNob3Blbi4xMDA2NTc4IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5pbnRlY2hvcGVuLmNvbS9jaXRhdGlvbi1wZGYtdXJsLzExODc4MTQiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMS0xNSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IlZldGVyaW5hcnkgbWVkaWNpbmUgYW5kIHNjaWVuY2UiLCJpbXBhY3RGYWN0b3IiOiIwLjk5In0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkludGVjaE9wZW4ifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjUyLCJjaXRlZEJ5Q291bnQiOjF9fSwicXVvdGUiOiJXYXRlciBRdWFsaXR5IENoYW5nZXMg4oCiIENsYXJpdHkgYW5kIEFsZ2FsIEJsb29tczogV2F0ZXIgY2xhcml0eSBtYXkgZGVjcmVhc2Ugc2xpZ2h0bHkgZHVlIHRvIHRoZSBpbml0aWFsIGFsZ2FsIGJsb29tcy4gSG93ZXZlciwgdGhpcyBwaGFzZSBpcyBvZnRlbiBzdGlsbCBjaGFyYWN0ZXJpemVkIGJ5IHJlbGF0aXZlbHkgZ29vZCB3YXRlciBxdWFsaXR5LiDigKIgQmlvZGl2ZXJzaXR5OiBUaGUgZWNvc3lzdGVtIG1pZ2h0IHN0aWxsIHN1cHBvcnQgYSBkaXZlcnNlIHJhbmdlIG9mIGFxdWF0aWMgb3JnYW5pc21zLCBpbmNsdWRpbmcgZmlzaCBhbmQgaW52ZXJ0ZWJyYXRlcy4gMy4xLjQuMiBQaGFzZSAyOiBNZXNvdHJvcGhpYyB0byBldXRyb3BoaWMgdHJhbnNpdGlvbiAxLiBOdXRyaWVudCBJbWJhbGFuY2Ug4oCiIFBob3NwaG9ydXMgdnMuIE5pdHJvZ2VuOiBBcyBhZ3JpY3VsdHVyYWwgcHJhY3RpY2VzIGludGVuc2lmeSBhbmQgdXJiYW5pemF0aW9uIGluY3JlYXNlcywgcGhvc3Bob3J1cyBpbnB1dHMgKGUuZy4sIGZyb20gZmVydGlsaXplcnMgYW5kIGRldGVyZ2VudHMpIG9mdGVuIGV4Y2VlZCB0aG9zZSBvZiBuaXRyb2dlbi4gVGhpcyBpbWJhbGFuY2UgY2FuIGRyaXZlIHNpZ25pZmljYW50IGNoYW5nZXMgaW4gd2F0ZXIgY2hlbWlzdHJ5IGFuZCBlY29zeXN0ZW0gZHluYW1pY3MuIDIuIEFsZ2FsIEJsb29tIEludGVuc2lmaWNhdGlvbiDigKIgQ3lhbm9iYWN0ZXJpYSBEb21pbmFuY2U6IEV4Y2VzcyBwaG9zcGhvcnVzIGNhbiBsZWFkIHRvIHRoZSBkb21pbmFuY2Ugb2YgY3lhbm9iYWN0ZXJpYSAoYmx1ZS1ncmVlbiBhbGdhZSksIHdoaWNoIGNhbiBvdXRjb21wZXRlIG90aGVyIGFsZ2FlIGFuZCBhcXVhdGljIHBsYW50cy4gVGhlc2UgYmxvb21zIGNhbiBiZSB0b3hpYyBhbmQgaGFybWZ1bCB0byBhcXVhdGljIGxpZmUuIOKAoiBIeXBveGlhOiBUaGUgZGVjb21wb3NpdGlvbiBvZiBsYXJnZSBhbGdhbCBibG9vbXMgY29uc3VtZXMgb3h5Z2VuLCBsZWFkaW5nIHRvIGh5cG94aWMgKGxvdyBveHlnZW4pIG9yIGFub3hpYyAobm8gb3h5Z2VuKSBjb25kaXRpb25zLCBwYXJ0aWN1bGFybHkgaW4gZGVlcGVyIHdhdGVycy4gMy4gV2F0ZXIgUXVhbGl0eSBEZXRlcmlvcmF0aW9uIOKAoiBEZWNyZWFzZWQgQ2xhcml0eTogV2F0ZXIgY2xhcml0eSBzaWduaWZpY2FudGx5IGRlY3JlYXNlcyBkdWUgdG8gZGVuc2UgYWxnYWwgZ3Jvd3RoLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo2Ny4yNjEwMDE1ODY5MTQwNiwibGVmdCI6MTUuNTgzNDU4OTAwNDUxNjYsIndpZHRoIjoyMC40NjA2MTg5NzI3NzgzMiwiaGVpZ2h0IjoxLjkwNTE0ODYyNTM3Mzg0MDN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NzAuNTEzMDMxMDA1ODU5MzgsImxlZnQiOjE3LjM5ODIxNjI0NzU1ODU5NCwid2lkdGgiOjY2LjQwMzU0OTE5NDMzNTk0LCJoZWlnaHQiOjEuOTA1MTU3MDg5MjMzMzk4NH19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo3Mi4xMzkwNDU3MTUzMzIwMywibGVmdCI6MTkuMDg4MzU2MDE4MDY2NDA2LCJ3aWR0aCI6NjUuNzg4NTIwODEyOTg4MjgsImhlaWdodCI6My41Mjg0NjA1MDI2MjQ1MTE3fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjc3LjAxNzA5NzQ3MzE0NDUzLCJsZWZ0IjoxNy4zOTgyMTYyNDc1NTg1OTQsIndpZHRoIjo2Mi4wOTQ1NzAxNTk5MTIxMSwiaGVpZ2h0IjoxLjkwNTE1NzA4OTIzMzM5ODR9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NzguNjQzMTEyMTgyNjE3MTksImxlZnQiOjE5LjA4ODM1NjAxODA2NjQwNiwid2lkdGgiOjM2LjcyNTMxMTI3OTI5Njg3NSwiaGVpZ2h0IjoxLjkwMjQ0NDI0MzQzMTA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6ODEuODk1MTMzOTcyMTY3OTcsImxlZnQiOjEzLjI0Njg1OTU1MDQ3NjA3NCwid2lkdGgiOjQwLjM5MTM5NTU2ODg0NzY1NiwiaGVpZ2h0IjoxLjkwNTE1NzA4OTIzMzM5ODR9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6ODUuMTQ3MTYzMzkxMTEzMjgsImxlZnQiOjE1LjU4MzQ1ODkwMDQ1MTY2LCJ3aWR0aCI6MTcuNjAyMTc4NTczNjA4NCwiaGVpZ2h0IjoxLjkwNTE1NzA4OTIzMzM5ODR9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6ODguMzk5MTkyODEwMDU4NiwibGVmdCI6MTcuMzk4MjE2MjQ3NTU4NTk0LCJ3aWR0aCI6NjUuMDA3MzQ3MTA2OTMzNiwiaGVpZ2h0IjoxLjkwNTE1NzA4OTIzMzM5ODR9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6OTAuMDI1MjA3NTE5NTMxMjUsImxlZnQiOjE5LjA4ODM1NjAxODA2NjQwNiwid2lkdGgiOjYxLjA1MTA1MjA5MzUwNTg2LCJoZWlnaHQiOjEuOTAyNDQ0MjQzNDMxMDkxM319LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjoxMC4zNTA0MTcxMzcxNDU5OTYsImxlZnQiOjIwLjg0MDgwNTA1MzcxMDkzOCwid2lkdGgiOjY2LjI3NzA1MzgzMzAwNzgxLCJoZWlnaHQiOjMuNTI4NDU1MjU3NDE1NzcxNX19LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjoxNS40OTk0Njc4NDk3MzE0NDUsImxlZnQiOjE3LjMzNTkwNjk4MjQyMTg3NSwid2lkdGgiOjIzLjUxOTYxNTE3MzMzOTg0NCwiaGVpZ2h0IjoxLjkwNTE0ODYyNTM3Mzg0MDN9fSx7InBhZ2VOdW1iZXIiOjEzLCJiYm94Ijp7InRvcCI6MTguNzUxNTAxMDgzMzc0MDIzLCJsZWZ0IjoxOS4xNTA2NjUyODMyMDMxMjUsIndpZHRoIjo2NC40Mzg3NjY0Nzk0OTIxOSwiaGVpZ2h0IjoxLjkwNTE0ODYyNTM3Mzg0MDN9fSx7InBhZ2VOdW1iZXIiOjEzLCJiYm94Ijp7InRvcCI6MjAuMzc3NTE3NzAwMTk1MzEyLCJsZWZ0IjoyMC44NDA4MDUwNTM3MTA5MzgsIndpZHRoIjo2MS4wNzQ0MjQ3NDM2NTIzNDQsImhlaWdodCI6My41Mjg0NTQ1NDIxNjAwMzR9fSx7InBhZ2VOdW1iZXIiOjEzLCJiYm94Ijp7InRvcCI6MjUuMjU1NTY3NTUwNjU5MTgsImxlZnQiOjE5LjE1MDY2NTI4MzIwMzEyNSwid2lkdGgiOjY4LjAyMzU1OTU3MDMxMjUsImhlaWdodCI6MS45MDUxNDg2MjUzNzM4NDAzfX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjI2Ljg4MTU4MjI2MDEzMTgzNiwibGVmdCI6MjAuODQwODA1MDUzNzEwOTM4LCJ3aWR0aCI6NjYuMDc2MjkzOTQ1MzEyNSwiaGVpZ2h0IjozLjUyODQ3MDk5MzA0MTk5Mn19LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjozMi4wMzA2NTEwOTI1MjkzLCJsZWZ0IjoxNy4zMzU5MDY5ODI0MjE4NzUsIndpZHRoIjoyNC42Nzc5NzY2MDgyNzYzNjcsImhlaWdodCI6MS45MDUxNDg2MjUzNzM4NDAzfX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjM1LjI4MjY4NDMyNjE3MTg3NSwibGVmdCI6MTkuMTUwNjY1MjgzMjAzMTI1LCJ3aWR0aCI6NjIuNDE5Nzg4MzYwNTk1NywiaGVpZ2h0IjoxLjkwNTE0ODYyNTM3Mzg0MDN9fSx7InBhZ2VOdW1iZXIiOjEzLCJiYm94Ijp7InRvcCI6MzYuOTA4Njk5MDM1NjQ0NTMsImxlZnQiOjIwLjg0MDgwNTA1MzcxMDkzOCwid2lkdGgiOjYuNjMzOTkzMTQ4ODAzNzExLCJoZWlnaHQiOjEuOTAyNDQwMTkwMzE1MjQ2Nn19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEzIn19XQ%3D%3D. Mitigation hinges on climate-resilient strategies: restoring riparian forests for shading and cooling (reducing temps by 2-3°C), advancing precision irrigation to curb runoff during extremes, and deploying aerators to break stratification (Shahbaz et al., 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1kIiwiZmFtaWx5IjoiU2hhaGJheiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBaGJhciIsImZhbWlseSI6IkFsYW0ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTW9oYW1tYWQgTWFzcm9vciIsImZhbWlseSI6IlphZmFyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1vaGFtbWVkIEFhc2lmIiwiZmFtaWx5IjoiU3VsYWltYW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW51cG1hIiwiZmFtaWx5IjoiS3VtYXJpIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkcuIEsuIiwiZmFtaWx5IjoiU2hhcm1hIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNoYWhsYSIsImZhbWlseSI6Illhc21pbiJ9fV0sImlkcyI6eyJpc3NuIjpbIjEyMTUtMTYxMCIsIjIwNjEtOTU4OCJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMjQ3OC9vcmh1LTIwMjMtMDAxNiIsIm9wZW5hbGV4IjoiVzQzODk1MTAwMzcifSwidGl0bGUiOiJDb21wYXJpc29uIG9mIGF2aWFuIGRpdmVyc2l0eSBiZXR3ZWVuIG1hbmFnZWQgYW5kIHVubWFuYWdlZCB3ZXRsYW5kcyBpbiBQYXRuYSwgQmloYXIsIEluZGlhIiwiYWJzdHJhY3QiOiJBYnN0cmFjdCBXZSBjb21wYXJlZCB0aGUgYmlyZCBkaXZlcnNpdHkgb2YgYSBtYW5hZ2VkIChSYWpkaGFuaSBKYWxhc2hheSkgYW5kIGFuIHVubWFuYWdlZCB3ZXRsYW5kIChNdXN0YWZhcHVyKSBpbiBQYXRuYSwgQmloYXIsIEluZGlhLiBXZSByZWNvcmRlZCBiaXJkIHNwZWNpZXMgYnkgdXNpbmcgcG9pbnQgY291bnRzIGFuZCBmaXhlZC1yb3V0ZSBtb25pdG9yaW5nLiBBdCBSYWpkaGFuaSBKYWxhc2hheSwgYSB0b3RhbCBvZiA3MyBzcGVjaWVzIHdlcmUgcmVjb3JkZWQsIG9mIHdoaWNoIDE1IHNwZWNpZXMgd2VyZSB3aW50ZXIgdmlzaXRvcnMuIFR3byBzcGVjaWVzIHdlcmUgaW4gdGhlIE5lYXItdGhyZWF0ZW5lZCBjYXRlZ29yeTogRmVycnVnaW5vdXMgRHVjayAoQXl0aHlhIG55cm9jYSkgYW5kIEFsZXhhbmRyaW5lIFBhcmFrZWV0IChQc2l0dGFjdWxhIGV1cGF0cmlhKSAsIHdoaWxlIHRoZSBvdGhlcnMgd2VyZSBvZiBsZWFzdCBjb25jZXJuLiBBdCBNdXN0YWZhcHVyIHdldGxhbmQsIDY3IHNwZWNpZXMgd2VyZSByZWNvcmRlZCB3aXRoIDExIHNwZWNpZXMgYXMgd2ludGVyIHZpc2l0b3JzLiBTaW1wc29u4oCZcyBzcGVjaWVzIGRpdmVyc2l0eSBpbmRleCB3YXMgMC43OCBhdCBSYWpkaGFuaSBKYWxhc2hheSwgYW5kIDAuODEgYXQgTXVzdGFmYXB1ciB3ZXRsYW5kLiBUaGUgbnVtYmVyIG9mIHNwZWNpZXMgb2Ygd2FkZXJzIHdhcyAxNyBhdCBNdXN0YWZhcHVyIHdldGxhbmQgYW5kIDExIGF0IFJhamRoYW5pIEphbGFzaGF5LiBUaGUgdG90YWwgbnVtYmVyIG9mIExlc3NlciBXaGlzdGxpbmctZHVjayAoRGVuZHJvY3lnbmEgamF2YW5pY2EpICwgYSByZXNpZGVudCBiaXJkIHdhcyBhbG1vc3QgZG91YmxlICg1MjYpIGF0IE11c3RhZmFwdXIgd2V0bGFuZCB0aGFuIHRoYXQgYXQgUmFqZGhhbmkgSmFsYXNoYXkgKDIzNCkuIFRoZSBuaXRyYXRlIGxldmVsIHdhcyB0d2ljZSBoaWdoZXIgYW5kIHRoZSBwaG9zcGhhdGUgbGV2ZWwgd2FzIGZvdXIgdGltZXMgaGlnaGVyIGF0IE11c3RhZmFwdXIgd2V0bGFuZCB0aGFuIHRob3NlIGF0IFJhamRoYW5pIEphbGFzaGF5LiBUaGUgZGlmZmVyZW5jZSBpbiBzcGVjaWVzIG51bWJlciBhbmQgY29tcG9zaXRpb24gYmV0d2VlbiB0aGUgdHdvIHdldGxhbmRzIG1heSBiZSBhdHRyaWJ1dGVkIHRvIHRoZSBldXRyb3BoaWNhdGlvbiByZXN1bHRpbmcgZnJvbSBoaWdoIG5pdHJhdGUgYW5kIHBob3NwaGF0ZSBsZXZlbHMgYXQgTXVzdGFmYXB1ciB3ZXRsYW5kLiBVbmZvcnR1bmF0ZWx5LCB0aGUgTXVzdGFmYXB1ciB3ZXRsYW5kIGlzIHVuZGVyIHRocmVhdCBkdWUgdG8gaHVtYW4gaW5mbHVlbmNlcy4gVGhlIHByb3RlY3Rpb24gYW5kIHN1c3RhaW5hYmxlIG1hbmFnZW1lbnQgb2YgbmF0dXJhbCB3ZXRsYW5kcyBpcyByZXF1aXJlZCBmb3Igc2F2aW5nIHRoZSBiaW9kaXZlcnNpdHkgb2YgdGhlIGFyZWEuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjI0Nzgvb3JodS0yMDIzLTAwMTYiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vc2NpZW5kby5jb20vcGRmLzEwLjI0Nzgvb3JodS0yMDIzLTAwMTYiLCJpc3N1ZWREYXRlIjoiMjAyMy0xMi0wMSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6Ik9ybmlzIEh1bmdhcmljYSIsImlzc3VlIjoyLCJ2b2x1bWUiOjMxLCJpbXBhY3RGYWN0b3IiOiIwLjkyIn0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTMsImxhc3RQYWdlIjoyOH19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJIdW5nYXJpYW4gT3JuaXRob2xvZ2ljYWwgYW5kIE5hdHVyZSBDb25zZXJ2YXRpb24gU29jaWV0eSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MjcsImNpdGVkQnlDb3VudCI6MX19LCJxdW90ZSI6Ikhvd2V2ZXIsIGhpZ2ggbGV2ZWxzIG9mIGV1dHJvcGhpY2F0aW9uIG1heSBhbHNvIHRyaWdnZXIgdGhlIHByb2xpZmVyYXRpb24gb2YgZ3JlZW4gYWxnYWUgYW5kICBkZWNsaW5lIG9mIG1hY3JvcGh5dGVzIGxlYWRpbmcgdG8gYSBoaWdoLXR1cmJpZGl0eSBzdGF0ZSBvZiB3YXRlciwgaW4gd2hpY2ggZmV3IHNwZWNpZXMgY2FuICBzdXJ2aXZlIChTY2hlZmZlciBldCBhbC4gMTk5MykuIFRvIGFkZHJlc3MgdGhlIGV1dHJvcGhpY2F0aW9uIGFuZCBleGNlc3MgbnV0cmllbnQgaXNzdWVzIGluICBhcXVhdGljIGVjb3N5c3RlbXMsIHBoeXRvcmVtZWRpYXRpb24gb2ZmZXJzIGEgcHJvbWlzaW5nIHNvbHV0aW9uIChMaXUgZXQgYWwuIDIwMjEpLiBBcXVhdGljICBwbGFudHMgbGlrZSBkdWNrd2VlZHMsIHdhdGVyIGxldHR1Y2UgKFBpc3RpYSBzdHJhdGlvdGVzKSwgYW5kIHdhdGVyIGxpbGllcyBoYXZlIGRlbW9uc3RyYXRlZCAgdGhlaXIgYWJpbGl0eSB0byBjb21iYXQgZXV0cm9waGljYXRpb24gYnkgYWJzb3JiaW5nIGFuZCBhc3NpbWlsYXRpbmcgZXhjZXNzIHBob3NwaG9ydXMgYW5kICBuaXRyb2dlbiAoTmFoYXIgJiBIb3F1ZSAyMDIxKS4gSG93ZXZlciwgdGhlc2UgcGxhbnRzIGNhbiBzb21ldGltZXMgZ3JvdyB1bmNvbnRyb2xsYWJseSwgIHBvdGVudGlhbGx5IGNvdmVyaW5nIHRoZSB3YXRlciBzdXJmYWNlcy4gVG8gYXZvaWQgc3VjaCBzY2VuYXJpb3MsIHJlZ3VsYXIgbW9uaXRvcmluZyBpcyBlc3NlbnRpYWwgKExpdSBldCBhbC4gMjAyMSkuIFByb21vdGluZyBwdWJsaWMgYXdhcmVuZXNzIGlzIHRoZSBjb3JuZXJzdG9uZSBmb3IgbW9iaWxpemluZyBzdXBwb3J0IGFuZCBlbnN1cmluZyBlZmZlY3RpdmUgZXV0cm9waGljYXRpb24gbWFuYWdlbWVudC4gQ29udHJvbGxlZCBlcmFkaWNhdGlvbiBvZiBleGNlc3MgIGZsb3JhIHdoZW4gbmVjZXNzYXJ5IGlzIHZpdGFsIHRvIHByZXZlbnQgb3Zlcmdyb3d0aC4gVGhpcyBpbnRlZ3JhdGVkIGFwcHJvYWNoIGNhbiBlZmZlY3RpdmVseSAgcmVkdWNlIGV1dHJvcGhpY2F0aW9uIGluIHRoZSB3YXRlciBib2RpZXMgKFdhbmcgZXQgYWwuIDIwMjIpLiBFYWNoIHNwZWNpZXMgb2Ygd2V0bGFuZC1kZXBlbmRlbnQgYmlyZCBoYXMgYSB1bmlxdWUgYW5kIGNvbXBsZXggc2V0IG9mIG5lZWRzIGZvciB3ZXRsYW5kIGhhYml0YXRzIHRoYXQgbWFrZXMgaXQgZGlmZmljdWx0IHRvIGdlbmVyYWxpemUgYWJvdXQgaG93IGxvc3Mgb3IgZGVncmFkYXRpb24gb2Ygd2V0bGFuZHMgYWZmZWN0cyBiaXJkIHBvcHVsYXRpb25zLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo3OS4yOTU2MzkwMzgwODU5NCwibGVmdCI6MTAuMzAzMDA2MTcyMTgwMTc2LCJ3aWR0aCI6NzkuOTUyNzI4MjcxNDg0MzgsImhlaWdodCI6MS43MDkxOTc4Nzg4Mzc1ODU0fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjgxLjIxMjUzOTY3Mjg1MTU2LCJsZWZ0IjoxMC4zMDMwMDYxNzIxODAxNzYsIndpZHRoIjo3OS45NTkyODE5MjEzODY3MiwiaGVpZ2h0IjoxLjcwOTE5Nzg3ODgzNzU4NTR9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6ODMuMTI5NDQ3OTM3MDExNzIsImxlZnQiOjEwLjMwMzAwNjE3MjE4MDE3Niwid2lkdGgiOjc5Ljk0MjcyNjEzNTI1MzksImhlaWdodCI6MS43MDkxOTc4Nzg4Mzc1ODU0fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjg1LjA0NjM0ODU3MTc3NzM0LCJsZWZ0IjoxMC4zMDMwMDgwNzk1Mjg4MDksIndpZHRoIjo3OS45Mzk0NTMxMjUsImhlaWdodCI6MS43MDkxOTc4Nzg4Mzc1ODU0fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjg2Ljk2MzI1NjgzNTkzNzUsImxlZnQiOjEwLjMwMzAxMDk0MDU1MTc1OCwid2lkdGgiOjc5Ljk0MTg0ODc1NDg4MjgxLCJoZWlnaHQiOjEuNzA5MTk3ODc4ODM3NTg1NH19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo4OC44ODAxNjUxMDAwOTc2NiwibGVmdCI6MTAuMzAzMDEzODAxNTc0NzA3LCJ3aWR0aCI6NzkuOTM3NSwiaGVpZ2h0IjoxLjcwOTE5Nzg3ODgzNzU4NTR9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6OTAuNzk3MDY1NzM0ODYzMjgsImxlZnQiOjEwLjMwMzAxMzgwMTU3NDcwNywid2lkdGgiOjc5LjkzOTY1MTQ4OTI1NzgxLCJoZWlnaHQiOjEuNzA5MTk3ODc4ODM3NTg1NH19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo5Mi43MTM5NzM5OTkwMjM0NCwibGVmdCI6MTAuMzAzMDEzODAxNTc0NzA3LCJ3aWR0aCI6NzkuOTUwMzI1MDEyMjA3MDMsImhlaWdodCI6MS43MDkxOTc4Nzg4Mzc1ODU0fX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjEyLjIwNzk5MDY0NjM2MjMwNSwibGVmdCI6MTAuMzAzMDI0MjkxOTkyMTg4LCJ3aWR0aCI6NzkuOTUyNTE0NjQ4NDM3NSwiaGVpZ2h0Ijo3LjQ1OTkyMjc5MDUyNzM0NH19LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjoxOS44NzU2MjU2MTAzNTE1NjIsImxlZnQiOjEyLjEyMTMyNDUzOTE4NDU3LCJ3aWR0aCI6NzguMTE5MjI0NTQ4MzM5ODQsImhlaWdodCI6MS43MDkxOTc4Nzg4Mzc1ODU0fX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjIxLjc5MjUzMzg3NDUxMTcyLCJsZWZ0IjoxMC4zMDMwMjkwNjAzNjM3Nywid2lkdGgiOjc5Ljk0OTI0OTI2NzU3ODEyLCJoZWlnaHQiOjEuNzA5MTk3ODc4ODM3NTg1NH19LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjoyMy43MDk0MzY0MTY2MjU5NzcsImxlZnQiOjEwLjMwMzAyOTA2MDM2Mzc3LCJ3aWR0aCI6NzkuOTUyMjcwNTA3ODEyNSwiaGVpZ2h0IjoxLjcwOTE5Nzg3ODgzNzU4NTR9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIyNSJ9fV0%3D. Integrating early-warning systems via satellite thermal imaging and citizen monitoring can preempt blooms, as demonstrated in pilot programs curbing 30% of Chilika's HAB extent (Nair & Nayak, 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hdnlhIFZpa3JhbWFuIiwiZmFtaWx5IjoiTmFpciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQcmF0ZWVwIEt1bWFyIiwiZmFtaWx5IjoiTmF5YWsifX1dLCJpZHMiOnsiaXNzbiI6WyIyMjk2LTc3NDUiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzODkvZm1hcnMuMjAyMy4xMDg3Mjk2Iiwib3BlbmFsZXgiOiJXNDM4MjcyMzEzMyJ9LCJ0aXRsZSI6IlVuY292ZXJpbmcgd2F0ZXIgcXVhbGl0eSBhbmQgZXZhbHVhdGluZyB2dWxuZXJhYmlsaXRpZXMgb2Ygc21hbGwtc2NhbGUgZmlzaGVyaWVzIGluIENoaWxpa2EgTGFnb29uLCBJbmRpYSIsImFic3RyYWN0IjoiU21hbGwtc2NhbGUgZmlzaGVyaWVzIChTU0ZzKSBwbGF5IGEgc3Ryb25nIHJvbGUgaW4gc3VzdGFpbmluZyBtaWxsaW9ucyBvZiBsaXZlbGlob29kcywgZm9vZCBzZWN1cml0eSwgbnV0cml0aW9uLCBhbmQgaW5jb21lIGdsb2JhbGx5IGJ1dCB0aGUgZmlzaGVycyBlbmdhZ2VkIGluIHRoaXMgc2VjdG9yIHNpbXVsdGFuZW91c2x5IGV4cGVyaWVuY2UgaGlnaCBsZXZlbHMgb2YgdnVsbmVyYWJpbGl0eSBhbmQgcHJvY2Vzc2VzIG9mIG1hcmdpbmFsaXNhdGlvbi4gU2V2ZXJhbCBmYWN0b3JzIGFyZSBhdHRyaWJ1dGVkIHRvIHRoZSBtdWx0aWRpbWVuc2lvbmFsIHZ1bG5lcmFiaWxpdGllcyB0aGUgc21hbGwtc2NhbGUgZmlzaGVycyBleHBlcmllbmNlLCBzcGFubmluZyBib3RoIG5hdHVyYWwgKGUuZy4sIG5hdHVyYWwgZGlzYXN0ZXJzLCBlY29zeXN0ZW0gY2hhbmdlKSBhbmQgYW50aHJvcG9nZW5pYyAoZS5nLiwgcG9saWN5IGNoYW5nZSwgaHlkcm9sb2dpY2FsIGludGVydmVudGlvbnMsIGFxdWFjdWx0dXJlKSBwcmVzc3VyZXMuIFdoaWxlIHRoZXJlIGlzIG11Y2ggbGl0ZXJhdHVyZSBvbiB2YXJpb3VzIG5hdHVyYWwgYW5kIGh1bWFuIGRyaXZlcnMgb2YgdnVsbmVyYWJpbGl0eSBpbiBzbWFsbC1zY2FsZSBmaXNoZXJ5IGNvbW11bml0aWVzLCBhbiBhYnNlbmNlIG9mIHJlc2VhcmNoIGNvbm5lY3RpbmcgdnVsbmVyYWJpbGl0eSB3aXRoIHdhdGVyIHF1YWxpdHkgaXMgZXZpZGVudC4gRmlzaGVyIGNvbW11bml0aWVzIG9mdGVuIHRhbGsgYWJvdXQgZmlzaCBpbiByZWxhdGlvbiB0byB0aGUgaGVhbHRoIG9mIHRoZWlyIGFxdWF0aWMgaGFiaXRhdHMgd2hlcmVpbiB3YXRlciBxdWFsaXR5IGlzIHNlZW4gYXMgYSBrZXkgcGFyYW1ldGVyLiBUaGUgbGluayBiZXR3ZWVuIGhlYWx0aHkgZmlzaCBhbmQgZ29vZCBxdWFsaXR5IHdhdGVyIGhhcyBzaWduaWZpY2FudCBpbXBsaWNhdGlvbnMgZm9yIHN0cm9uZyBhbmQgdmlhYmxlIGZpc2hpbmcgY29tbXVuaXRpZXMuIFRoaXMgcGFwZXIgZXhhbWluZXMgdGhlc2UgbGlua3MgZnVydGhlciBieSBmb2N1c2luZyBvbiB0aGUgbmF0dXJlIG9mIHZ1bG5lcmFiaWxpdGllcyBjYXVzZWQgYnkgd2F0ZXIgcXVhbGl0eSBjaGFuZ2VzIGluIHRoZSBzbWFsbC1zY2FsZSBmaXNoZXJ5IHN5c3RlbSBvZiBDaGlsaWthIExhZ29vbiBpbiBJbmRpYS4gV2UgdW5kZXJ0YWtlIGRldGFpbGVkIGFuYWx5c2lzIG9mIHRoZSBpbnZhc2l2ZSBzaHJpbXAgYXF1YWN1bHR1cmUgYWN0aXZpdGllcyBhbmQgaHlkcm9sb2dpY2FsIGludGVydmVudGlvbnMgZm9yIG9wZW5pbmcgb2YgYSBsYWdvb24gaW5sZXQgd2l0aCB0aGUgQmF5IG9mIEJlbmdhbCBhcyB0d28gZG9taW5hbnQgZHJpdmVycyBhZHZlcnNlbHkgaW1wYWN0aW5nIHdhdGVyIHF1YWxpdHkgYW5kIGluY3JlYXNpbmcgdnVsbmVyYWJpbGl0aWVzIG9mIHRoZSBlbnRpcmUgc21hbGwtc2NhbGUgZmlzaGVyaWVzIHNvY2lhbC1lY29sb2dpY2FsIHN5c3RlbS4gT3VyIGFuYWx5c2lzIHN1Z2dlc3RzIHRoYXQgdGhlcmUgYXJlIHN0cm9uZyBpbnRlcmNvbm5lY3Rpb25zIGJldHdlZW4gY2hhbmdlcyBpbiB3YXRlciBxdWFsaXR5IGFuZCB0aGUgbGV2ZWxzIG9mIHZ1bG5lcmFiaWxpdGllcyBpbiB0aGUgU1NGcyBvZiBDaGlsaWthIExhZ29vbi4gUG9sbHV0YW50cyBzdWNoIGFzIHBlc3RpY2lkZXMsIGFuZCBvcmdhbmljIGNvbXBvdW5kcyBhY2N1bXVsYXRlIGluIGZpc2ggdGlzc3VlcyBhbmQgYWZmZWN0IHRoZWlyIGdyb3d0aCwgcmVwcm9kdWN0aW9uLCBhbmQgb3ZlcmFsbCBoZWFsdGguIFRoaXMgbGVkIHRvIGRlY2xpbmVzIGluIGZpc2ggcG9wdWxhdGlvbnMsIG1ha2luZyBpdCBtb3JlIGRpZmZpY3VsdCBmb3IgZmlzaGVycyB0byBtYWtlIGEgbGl2aW5nLiBJbiBhZGRpdGlvbiB0byBkaXJlY3QgaW1wYWN0cyBvbiBmaXNoIHBvcHVsYXRpb25zLCBwb29yIHdhdGVyIHF1YWxpdHkgYWxzbyBoYXMgaW5kaXJlY3QgZWZmZWN0cyBvbiB0aGUgc29jaWFsIGFuZCBlY29ub21pYyB2dWxuZXJhYmlsaXR5IG9mIFNTRnMuIEZvciBleGFtcGxlLCBjb250YW1pbmF0aW9uIG9mIHdhdGVyIHNvdXJjZXMgbGVkIHRvIHRoZSByZWR1Y2VkIG51bWJlciBvZiBmaXNoIHNwZWNpZXMgcmVkdWNpbmcgdGhlIGFtb3VudCBvZiB0aW1lIGZpc2hlcnMgY2FuIHNwZW5kIG9uIGZpc2hpbmcgYWN0aXZpdGllcy4gVGhpcyBhbHNvIGFmZmVjdGVkIHRoZSBtYXJrZXRhYmlsaXR5IG9mIGZpc2ggcHJvZHVjdHMsIHJlZHVjZWQgaW5jb21lIGFuZCBpbmNyZWFzZWQgcG92ZXJ0eS4gVG8gZnVsbHkgdW5kZXJzdGFuZCB0aGUgaW50ZXJjb25uZWN0aW9ucyBiZXR3ZWVuIHdhdGVyIHF1YWxpdHkgYW5kIHZ1bG5lcmFiaWxpdHkgaW4gU1NGcyBpbiBDaGlsaWthIExhZ29vbiwgaXQgaXMgaW1wb3J0YW50IHRvIGNvbnNpZGVyIGJvdGggZW52aXJvbm1lbnRhbCBhbmQgc29jaWFsIGZhY3RvcnMsIGFzIHdlbGwgYXMgdGhlIGNvbXBsZXggZmVlZGJhY2sgbG9vcHMgYmV0d2VlbiB0aGVzZSBmYWN0b3JzLiBUaGUgc3R1ZHkgaGVscHMgaW4gYnJpZGdpbmcgYSBjcnVjaWFsIGdhcCBpbiBvdXIgdW5kZXJzdGFuZGluZyBvZiB0aGUgcm9sZSBvZiB3YXRlciBxdWFsaXR5IGluIHZ1bG5lcmFiaWxpdHkgYW5hbHlzaXMgd2l0aGluIHJlc291cmNlIGRlcGVuZGVudCBjb21tdW5pdGllcy4gV2UgY29uY2x1ZGUgd2l0aCBrZXkgaW5zaWdodHMgb24gcG9zc2libGUgY29waW5nIHJlc3BvbnNlcyBhbmQgYWRhcHRpdmUgY2FwYWNpdHkgbmVjZXNzYXJ5IGZvciB0aGUgc21hbGwtc2NhbGUgZmlzaGVyaWVzIGNvbW11bml0aWVzIHRvIHRyYW5zaXRpb24gdG93YXJkIHZpYWJpbGl0eS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMzM4OS9mbWFycy4yMDIzLjEwODcyOTYiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vd3d3LmZyb250aWVyc2luLm9yZy9hcnRpY2xlcy8xMC4zMzg5L2ZtYXJzLjIwMjMuMTA4NzI5Ni9wZGYiLCJpc3N1ZWREYXRlIjoiMjAyMy0wNi0yOSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZyb250aWVycyBpbiBNYXJpbmUgU2NpZW5jZSIsInZvbHVtZSI6MTAsImltcGFjdEZhY3RvciI6IjQuMjgifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRnJvbnRpZXJzIE1lZGlhIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo1MSwiY2l0ZWRCeUNvdW50Ijo1fX0sInF1b3RlIjoiV2F0ZXIgcXVhbGl0eSBvZiBDaGlsaWthIExhZ29vbiBoYXMgZGVncmFkZWQgZHVlIHRvIGFudGhyb3BvZ2VuaWMgaW50ZXJ2ZW50aW9ucyBhbmQgbmF0dXJhbCBmb3JjZXMgb2YgY2hhbmdlLiBWYXJpb3VzIGFjdGl2aXRpZXMgaW4gdGhlIENoaWxpa2EgTGFnb29uLCBzdWNoIGFzIGluZHVzdHJpYWwgd2FzdGV3YXRlciBkaXNwb3NhbCwgc2V3YWdlIGR1bXBpbmcsIGFxdWFjdWx0dXJlLCBoeWRyb2xvZ2ljYWwgaW50ZXJ2ZW50aW9ucywgYW5kIGN5Y2xvbmVzLCBhcmUgcHJvZHVjaW5nIHNhbGluaXR5IGNoYW5nZXMsIHNlZGltZW50IGRlcG9zaXRpb24sIG51dHJpZW50IGVucmljaG1lbnQsIGV1dHJvcGhpY2F0aW9uLCBhbmQgZGVhZCB6b25lcywgYXMgc3RhdGVkLiBGaXNoIHN1cnZpdmFsIGlzIGF0IHJpc2sgYXMgYSByZXN1bHQgb2YgdGhlIHNpZ25pZmljYW50IHJlZHVjdGlvbiBpbiBlbnZpcm9ubWVudGFsIGNpcmN1bXN0YW5jZXMsIHdoaWNoIGRpbWluaXNoZXMgZmlzaGVybWVu4oCZcyBpbmNvbWUgYW5kIHB1c2hlcyB0aGVtIG91dCBvZiBzb2NpZXR5LiBUaGVzZSBjYXVzZXMgYWZmZWN0aW5nIHRoZSB3YXRlciBxdWFsaXR5IG9mIHRoZSBMYWdvb24gZGVwaWN0IG1hbnkgZmFjZXMgb2YgdnVsbmVyYWJpbGl0eSBpbiBTU0YgY29tbXVuaXRpZXMuIEFzIHRoZSBDaGlsaWthIExhZ29vbiBlbnZpcm9ubWVudCBpcyBleHBlY3RlZCB0byB1bmRlcmdvIGluY3JlYXNpbmdseSBkcmFzdGljIGNoYW5nZXMgKHN1Y2ggYXMgY2F0Y2ggY29tcG9zaXRpb24sIGNhdGNoIGNhcGFjaXR5LCBiaW9kaXZlcnNpdHkgdmFyaWF0aW9ucywgd2F0ZXIgcXVhbGl0eSwgYW5kIGZpc2hlcnkgcmV2ZW51ZXMpLCBpbnRlcm5hdGlvbmFsIHBvbGljeSBoYXMgYmVjb21lIG1vcmUgaW50ZXJlc3RlZCBpbiBhZGFwdGF0aW9uIG1lYXN1cmVzLiBHaXZlbiBob3cgZGl2ZXJzZSBTU0YgZWNvc3lzdGVtcyBhcmUgZXhwZWN0ZWQgdG8gYmUgYmV0d2VlbiBjb250aW5lbnRzLCB0aGVyZSBpcyBubyDigJxvbmUgc2l6ZSBmaXRzIGFsbOKAnSBhcHByb2FjaC4gVGhlIHBhcGVyIG91dGxpbmVkIHNldmVyYWwgY29waW5nIGFuZCBhZGFwdGF0aW9uIG1lY2hhbmlzbXMgdG8gaW1wcm92ZSB3YXRlciBxdWFsaXR5IGFuZCBjb250aW51ZSBmaXNoaW5nIGluIGEgc3VzdGFpbmFibGUgd2F5LiBBZGFwdGF0aW9uIHN0cmF0ZWdpZXMgY2FuIG9jY2FzaW9uYWxseSBoYXZlIHVuZm9yZXNlZW4gY29uc2VxdWVuY2VzIGZvciBmaXNoZXJpZXMgYW5kIGZpc2hpbmcgY29tbXVuaXRpZXMuIEFueSB1bmV4cGVjdGVkIGNvbnNlcXVlbmNlcyBvZiBhZGFwdGF0aW9uIGNhbiBiZSBtaXRpZ2F0ZWQgdGhyb3VnaCBsb25nLXRlcm0gcGxhbm5pbmcgYW5kIGlkZW50aWZpY2F0aW9uIG9mIGZ1dHVyZSBvcHRpb25zIHRocm91Z2ggc2NlbmFyaW8gYW5hbHlzaXMuIFRoZSBmaW5kaW5ncyBvZiB0aGlzIHBhcGVyIGNhbiBiZSB1dGlsaXplZCB0byBkZXZlbG9wIHBvbGljaWVzIHRvIHByb3RlY3Qgd2F0ZXIgcXVhbGl0eSBhbmQgU1NGcyBpbiBhIHZhcmlldHkgb2YgY29hc3RhbCBsYWdvb24gc2V0dGluZ3MuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjIyLjI4NTAzNjA4NzAzNjEzMywibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ5MzA1MzQzNjI3OTMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjIzLjc3MzUxOTUxNTk5MTIxLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDgzMzY0MTA1MjI0NjEsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjI1LjI1NTU4NDcxNjc5Njg3NSwibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ4OTU1OTE3MzU4Mzk4NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjYuNzQ0MDY4MTQ1NzUxOTUzLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNTA2ODA1NDE5OTIxODc1LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyOC4yMjYxMzMzNDY1NTc2MTcsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40OTA0NjMyNTY4MzU5NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjkuNzE0NjE2Nzc1NTEyNjk1LCJsZWZ0Ijo4LjMzMTU5MjU1OTgxNDQ1Mywid2lkdGgiOjQwLjQ5NDA5ODY2MzMzMDA4LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozMS4xOTY2ODM4ODM2NjY5OTIsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40OTQzOTIzOTUwMTk1MywiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MzIuNjg1MTY1NDA1MjczNDQsImxlZnQiOjguMzMxNTk2Mzc0NTExNzE5LCJ3aWR0aCI6NDAuNDg3NDQ1ODMxMjk4ODMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM0LjE2NzIzMjUxMzQyNzczNCwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40OTc3NDU1MTM5MTYwMTYsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM1LjY1NTcxNTk0MjM4MjgxLCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ5ODYzODE1MzA3NjE3LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozNy4xMzc3ODMwNTA1MzcxMSwibGVmdCI6MTAuNjg1MTg2Mzg2MTA4Mzk4LCJ3aWR0aCI6MzguMTQxMTA1NjUxODU1NDcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM4LjYyNjI2NjQ3OTQ5MjE5LCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4NzE1MjA5OTYwOTM3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDAuMTA4MzMzNTg3NjQ2NDg0LCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4OTg1NjcxOTk3MDcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQxLjU5NjgxMzIwMTkwNDMsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDg4NjY2NTM0NDIzODMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQzLjA3ODg4MDMxMDA1ODU5NCwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40ODE4NjExMTQ1MDE5NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDQuNTY3MzYzNzM5MDEzNjcsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDgzMDU4OTI5NDQzMzYsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQ2LjA0OTQzMDg0NzE2Nzk3LCJsZWZ0Ijo4LjMzMzEwODkwMTk3NzUzOSwid2lkdGgiOjQwLjUwMDc0NzY4MDY2NDA2LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo0Ny41Mzc5MTQyNzYxMjMwNSwibGVmdCI6OC4zMzMxMDg5MDE5Nzc1MzksIndpZHRoIjo0MC40OTU2MDU0Njg3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDkuMDE5OTgxMzg0Mjc3MzQ0LCJsZWZ0Ijo4LjMzMzEwNjA0MDk1NDU5LCJ3aWR0aCI6NDAuNTA2ODE2ODY0MDEzNjcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUwLjUwODQ2NDgxMzIzMjQyLCJsZWZ0Ijo4LjMzMzEwNjA0MDk1NDU5LCJ3aWR0aCI6NDAuNDg4NTA2MzE3MTM4NjcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUxLjk5MDUyODEwNjY4OTQ1LCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDk3NzI2NDQwNDI5NjksImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUzLjQ3OTAxMTUzNTY0NDUzLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6MzMuMTU5NDQ2NzE2MzA4NTk0LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo1NC45NjEwNzg2NDM3OTg4MywibGVmdCI6MTAuNjgzNjcwMDQzOTQ1MzEyLCJ3aWR0aCI6MzguMTM2NTk2Njc5Njg3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NTYuNDQ5NTYyMDcyNzUzOTA2LCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDg3MTQ0NDcwMjE0ODQ0LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEzIn19XQ%3D%3D. Addressing this climate-eutrophication nexus demands urgent, adaptive governance to preserve India's lakes before irreversible dead zones prevail. This self-perpetuating cycle is driven by cyanobacterial dominance, which enhances localized surface warming and thermal stratification while promoting greenhouse gas release and nutrient remineralization during bloom collapse (Meerhoff et al., 2022)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik0uIiwiZmFtaWx5IjoiTWVlcmhvZmYifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSi4iLCJmYW1pbHkiOiJBdWRldCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJULkEuIiwiZmFtaWx5IjoiRGF2aWRzb24ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTC4gRGUiLCJmYW1pbHkiOiJNZWVzdGVyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlMuIiwiZmFtaWx5IjoiSGlsdCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTLiIsImZhbWlseSI6Iktvc3RlbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJaYWlodWEiLCJmYW1pbHkiOiJMaXUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTi4iLCJmYW1pbHkiOiJNYXp6ZW8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSGFucyBXLiIsImZhbWlseSI6IlBhZXJsIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik0uIiwiZmFtaWx5IjoiU2NoZWZmZXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiRS4iLCJmYW1pbHkiOiJKZXBwZXNlbiJ9fV0sImlkcyI6eyJpc3NuIjpbXSwiaXNibiI6W10sImRvaSI6IjEwLjE3NjE1Lzd2ZTUtdzAxNiIsIm9wZW5hbGV4IjoiVzQzMTg0MTU1NzciLCJvcGVuQWNjZXNzSWQiOiI2ODM1ODI5NzkxODE2MDA3MjA0In0sInRpdGxlIjoiRmVlZGJhY2sgYmV0d2VlbiBjbGltYXRlIGNoYW5nZSBhbmQgZXV0cm9waGljYXRpb246IHJldmlzaXRpbmcgdGhlIGFsbGllZCBhdHRhY2sgY29uY2VwdCBhbmQgaG93IHRvIHN0cmlrZSBiYWNrIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjE3NjE1Lzd2ZTUtdzAxNiIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9jZHIubGliLnVuYy5lZHUvZG93bmxvYWRzL2g5ODlyYzkzOCIsImlzc3VlZERhdGUiOiIyMDIyLTAxLTAxIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiQ2Fyb2xpbmEgRGlnaXRhbCBSZXBvc2l0b3J5IChVbml2ZXJzaXR5IG9mIE5vcnRoIENhcm9saW5hIGF0IENoYXBlbCBIaWxsKSIsImltcGFjdEZhY3RvciI6IjAuMTAifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiVW5pdmVyc2l0eSBvZiBOb3J0aCBDYXJvbGluYSBhdCBDaGFwZWwgSGlsbCJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MCwiY2l0ZWRCeUNvdW50IjozfX0sInF1b3RlIjoiV2l0aCBjbGltYXRlIHdhcm1pbmcsIHRoZSBkaXJlY3QgYW5kIGluZGktcmVjdCBlZmZlY3RzIG9mIGluY3JlYXNlZCBsYWtlIHRlbXBlcmF0dXJlcyB3aWxsIGxpa2VseSBkcml2ZSBjaGFuZ2VzIGluIGN5YW5vYmFjdGVyaWFsIHRheGEgYW5kIHRoZWlyIHByb2R1Yy10aW9uIG9mIGN5YW5vYmFjdGVyaWFsIHRveGlucywgbGlrZWx5IGluY2x1ZGluZyBwcm9tb3QtaW5nIHRoZSBzZWxlY3Rpb24gb2YgYSBmZXcgTWljcm9jeXN0aXMgc3BlY2llcyBvciBzdHJhaW5zIHByb2R1Y2luZyBoaWdobHkgdG94aWMgbWljcm9jeXN0aW4tTFIgKE1hbnR6b3VraSBldCBhbC4gMjAxOCkuIERlc3BpdGUgdmFyaWF0aW9ucyBhbW9uZyBsYWtlcyBpbiBhIHdhdGVyLXNjYXBlLCBhIHJlZ2lvbmFsIGluY3JlYXNlIGluIG1pY3JvY3lzdGluIG1heGltYSBhbmQgZHVyYXRpb24gb2YgYmxvb21zIGhhcyBhbHNvIGJlZW4gY2xlYXJseSBleHBsYWluZWQgYnkgaW5jcmVhc2luZyB0ZW1wZXJhdHVyZSBpbiBjZW50cmFsIE5vcnRoIEFtZXJpY2EgKEhheWVzIGV0IGFsLiAyMDIwKSwgd2l0aCBpbmNyZWFzaW5nIHJpc2tzIHRvIGh1bWFuIGhlYWx0aC4gR3Jvd2luZyBldmlkZW5jZSBpbmRpY2F0ZXMgdGhhdCBjeWFub2JhY3RlcmlhbCBkb21pbmFuY2Ugd2lsbCBleGVydCBhIHBvc2l0aXZlIGZlZWRiYWNrIHdpdGggY2xpbWF0ZSB3YXJtaW5nIChIYXZlbnMgZXQgYWwuIDIwMTYsIFBhZXJsIGV0IGFsLiAyMDIwKS4gTWFueSBjeWFub2JhY3RlcmlhbCBzcGVjaWVzIHByb2xpZmVyYXRlIG9uIHRoZSBsYWtlIHN1cmZhY2UsIGFic29yYmluZyBsaWdodCwgZW5oYW5jaW5nIGxvY2FsaXplZCB0ZW1wZXJhdHVyZSBpbmNyZWFzZXMsIGFuZCBpbmNyZWFzaW5nIHRoZXJtYWwgc3RyYXRpZmljYXRpb24gcG90ZW50aWFsIChLYWhydSBldCBhbC4gMTk5MykuIEN5YW5vYmFjdGVyaWEgcHJvbW90ZSB0aGUgcmVsZWFzZSBvZiBHSEdzLCBwYXJ0aWN1bGFybHkgQ0g0IChZYW4gZXQgYWwuIDIwMTcpLCBkdXJpbmcgdGhlIGRlY29tcG9zaXRpb24gZm9sbG93aW5nIHRoZSBjb2xsYXBzZSBvZiBhIGJsb29tLiBDeWFub2JhY3RlcmlhIGFsc28gcHJvbW90ZSByZW1pbmVyYWxpemF0aW9uIG9mIG51dHJpLWVudHMgZHVyaW5nIGJsb29tcywgbGVhZGluZyB0byBtdWx0aXBsZSBibG9vbXMgaW4gYSBzaW5nbGUgc2Vhc29uLiBVbmRlciBldXRyb3BoaWMgYW5kIHdhcm1pbmcgY29uZGktdGlvbnMsIGN5YW5vYmFjdGVyaWEgZG9taW5hbmNlIG1heSB0aHVzIGJlIGEgc2VsZi1wZXJwZXR1YXRpbmcgcHJvcGhlY3kgKEZpZy4gMiBhbmQgMykuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6NjcuMDYzNDAwMjY4NTU0NjksImxlZnQiOjUxLjQ3ODgwMTcyNzI5NDkyLCJ3aWR0aCI6MzguNjczNDIzNzY3MDg5ODQ0LCJoZWlnaHQiOjEuNDcxMTAxNTIyNDQ1Njc4N319LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjY4LjY5ODU0NzM2MzI4MTI1LCJsZWZ0Ijo1MS40Nzg4MDE3MjcyOTQ5Miwid2lkdGgiOjM4LjY3OTQ5Njc2NTEzNjcyLCJoZWlnaHQiOjEuNDcxMTAxNTIyNDQ1Njc4N319LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjcwLjMzMzY4NjgyODYxMzI4LCJsZWZ0Ijo1MS40Nzg4MDE3MjcyOTQ5Miwid2lkdGgiOjM4LjY3NzU4MTc4NzEwOTM3NSwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo3MS45Njg4MjYyOTM5NDUzMSwibGVmdCI6NTEuNDc4ODAxNzI3Mjk0OTIsIndpZHRoIjozOC42NzkxMDM4NTEzMTgzNiwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo3My42MTA1NzI4MTQ5NDE0LCJsZWZ0Ijo1MS40Nzg4MDE3MjcyOTQ5Miwid2lkdGgiOjM4LjY3NDQyNzAzMjQ3MDcsImhlaWdodCI6MS40NzM3NDY4OTU3OTAxfX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6NzUuMjQ3MDM5Nzk0OTIxODgsImxlZnQiOjUxLjQ3ODgwOTM1NjY4OTQ1LCJ3aWR0aCI6MzguNjc0MjYzMDAwNDg4MjgsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6NzYuODgyMTg2ODg5NjQ4NDQsImxlZnQiOjUxLjQ3ODgwOTM1NjY4OTQ1LCJ3aWR0aCI6MzguNjgxMTcxNDE3MjM2MzMsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6NzguNTI1MjUzMjk1ODk4NDQsImxlZnQiOjUxLjQ3ODgwOTM1NjY4OTQ1LCJ3aWR0aCI6MzguNjc1NjYyOTk0Mzg0NzY2LCJoZWlnaHQiOjEuNDcxMTAxNTIyNDQ1Njc4N319LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjgwLjE2MDQwMDM5MDYyNSwibGVmdCI6NTEuNDc4ODA5MzU2Njg5NDUsIndpZHRoIjozOC42ODIwMjk3MjQxMjEwOTQsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODEuNzk1NTM5ODU1OTU3MDMsImxlZnQiOjUxLjQ3ODgwOTM1NjY4OTQ1LCJ3aWR0aCI6MzguNjc0OTgzOTc4MjcxNDg0LCJoZWlnaHQiOjEuNDcxMTAxNTIyNDQ1Njc4N319LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjgzLjQzODYxMzg5MTYwMTU2LCJsZWZ0Ijo1MS40Nzg4MDkzNTY2ODk0NSwid2lkdGgiOjM4LjY3NjE4NTYwNzkxMDE1NiwiaGVpZ2h0IjoxLjQ4Njk3MzUyNDA5MzYyOH19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjg1LjA4OTYzMDEyNjk1MzEyLCJsZWZ0Ijo1MS40Nzg4MDkzNTY2ODk0NSwid2lkdGgiOjQuNjgyODA1MDYxMzQwMzMyLCJoZWlnaHQiOjEuNDcxMTAxNTIyNDQ1Njc4N319LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjg2LjcyNDc3NzIyMTY3OTY5LCJsZWZ0Ijo1My40NDExNzczNjgxNjQwNiwid2lkdGgiOjYuMTQwMzU1NTg3MDA1NjE1LCJoZWlnaHQiOjEuNDcxMTAxNTIyNDQ1Njc4N319LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjg2LjcyNDg5MTY2MjU5NzY2LCJsZWZ0Ijo2MC45OTQ4NzY4NjE1NzIyNjYsIndpZHRoIjo2LjAzNjk5MjU0OTg5NjI0LCJoZWlnaHQiOjEuNDcxMTAxNTIyNDQ1Njc4N319LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjg2LjcyNDg5MTY2MjU5NzY2LCJsZWZ0Ijo2OC40NDY1MjU1NzM3MzA0Nywid2lkdGgiOjYuMTUwNzA1ODE0MzYxNTcyLCJoZWlnaHQiOjEuNDcxMTAxNTIyNDQ1Njc4N319LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjg2LjcyNDg5MTY2MjU5NzY2LCJsZWZ0Ijo3Ni4wMDk4MjY2NjAxNTYyNSwid2lkdGgiOjIuNzI3MzMzMzA3MjY2MjM1NCwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo4Ni43MjQ4OTE2NjI1OTc2NiwibGVmdCI6ODAuMTQ5NTgxOTA5MTc5NjksIndpZHRoIjoxMC4wMDgyMzQ5Nzc3MjIxNjgsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODguMzYwMDM4NzU3MzI0MjIsImxlZnQiOjUxLjQ3NzY1NzMxODExNTIzNCwid2lkdGgiOjM4LjY3MjA1NDI5MDc3MTQ4NCwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo5MC4wMDMxMTI3OTI5Njg3NSwibGVmdCI6NTEuNDc3NjU3MzE4MTE1MjM0LCJ3aWR0aCI6MzguNjc2NDk4NDEzMDg1OTQsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6OTEuNjM4MjUyMjU4MzAwNzgsImxlZnQiOjUxLjQ3NzY1MzUwMzQxNzk3LCJ3aWR0aCI6MzguNjc1ODM0NjU1NzYxNzIsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo5LCJiYm94Ijp7InRvcCI6Ni40Njg5MzU5NjY0OTE2OTksImxlZnQiOjkuODQwMTMzNjY2OTkyMTg4LCJ3aWR0aCI6MzguNjg2MzI1MDczMjQyMTksImhlaWdodCI6MTYuMjEyMTI1Nzc4MTk4MjQyfX1dfSwiY2h1bmtJZCI6ImQ1YjkwNzAxLTU3ODQtNDg5MS05NDc5LTBhYjk5NzlhYzRhYyIsImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjkifX1d. The synergistic combination of anthropogenic nutrient loading, rising temperatures, enhanced vertical stratification, increased residence time and more extreme climatic conditions overall will favour cyanobacterial dominance in a wide range of aquatic ecosystems (Paerl et al., 2019)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhhbnMgVy4iLCJmYW1pbHkiOiJQYWVybCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLYXJsIEUuIiwiZmFtaWx5IjoiSGF2ZW5zIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hdGhhbiBTLiIsImZhbWlseSI6IkhhbGwifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiVGltb3RoeSBHLiIsImZhbWlseSI6Ik90dGVuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1lbmd5dWFuIiwiZmFtaWx5IjoiWmh1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhhaSIsImZhbWlseSI6Ilh1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikd1YW5nd2VpIiwiZmFtaWx5IjoiWmh1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkJvcWlhbmciLCJmYW1pbHkiOiJRaW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIxMzIzLTE2NTAiLCIxNDQ4LTYwNTkiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwNzEvbWYxODM5MiIsIm1hZyI6IjI5NDIzMjE1NDIiLCJvcGVuYWxleCI6IlcyOTQyMzIxNTQyIiwib3BlbkFjY2Vzc0lkIjoiNjU3NTI1NDM0OTg3OTgyNDM2NCJ9LCJ0aXRsZSI6Ik1pdGlnYXRpbmcgYSBnbG9iYWwgZXhwYW5zaW9uIG9mIHRveGljIGN5YW5vYmFjdGVyaWFsIGJsb29tczogY29uZm91bmRpbmcgZWZmZWN0cyBhbmQgY2hhbGxlbmdlcyBwb3NlZCBieSBjbGltYXRlIGNoYW5nZSIsImFic3RyYWN0IjoiTWFuYWdpbmcgYW5kIG1pdGlnYXRpbmcgdGhlIGdsb2JhbCBleHBhbnNpb24gb2YgdG94aWMgY3lhbm9iYWN0ZXJpYWwgaGFybWZ1bCBhbGdhbCBibG9vbXMgKEN5YW5vSEFCcykgaXMgYSBtYWpvciBjaGFsbGVuZ2UgZmFjaW5nIHJlc2VhcmNoZXJzIGFuZCB3YXRlciByZXNvdXJjZSBtYW5hZ2Vycy4gVmFyaW91cyBhcHByb2FjaGVzLCBpbmNsdWRpbmcgbnV0cmllbnQgbG9hZCByZWR1Y3Rpb24sIGFydGlmaWNpYWwgbWl4aW5nIGFuZCBmbHVzaGluZywgb21uaXZvcm91cyBmaXNoIHJlbW92YWwsIGFsZ2FlY2lkZSBhcHBsaWNhdGlvbnMgYW5kIHNlZGltZW50IGRyZWRnaW5nLCBoYXZlIGJlZW4gdXNlZCB0byByZWR1Y2UgYmxvb20gb2NjdXJyZW5jZXMuIEhvd2V2ZXIsIG1hbmFnZXJzIG5vdyBmYWNlIHRoZSBhZGRpdGlvbmFsIGNoYWxsZW5nZSBvZiBoYXZpbmcgdG8gYWRkcmVzcyB0aGUgZWZmZWN0cyBvZiBjbGltYXRlIGNoYW5nZSBvbiB3YXRlcnNoZWQgaHlkcm9sb2dpY2FsIGFuZCBudXRyaWVudCBsb2FkIGR5bmFtaWNzLCB3YXRlciB0ZW1wZXJhdHVyZSwgbWl4aW5nIHJlZ2ltZSBhbmQgaW50ZXJuYWwgbnV0cmllbnQgY3ljbGluZy4gUmlzaW5nIHRlbXBlcmF0dXJlcyBhbmQgaW5jcmVhc2luZyBmcmVxdWVuY2llcyBhbmQgbWFnbml0dWRlcyBvZiBleHRyZW1lIHdlYXRoZXIgZXZlbnRzLCBpbmNsdWRpbmcgdHJvcGljYWwgY3ljbG9uZXMsIGV4dHJhdHJvcGljYWwgc3Rvcm1zLCBmbG9vZHMgYW5kIGRyb3VnaHRzLCBhbGwgcHJvbW90ZSBDeWFub0hBQnMgYW5kIGFmZmVjdCB0aGUgZWZmaWNhY3kgb2YgZWNvc3lzdGVtIHJlbWVkaWF0aW9uIG1lYXN1cmVzLiBUaGVzZSBjbGltYXRpYyBjaGFuZ2VzIHdpbGwgbGlrZWx5IHJlcXVpcmUgc2V0dGluZyBzdHJpY3RlciBudXRyaWVudCAoaW5jbHVkaW5nIGJvdGggbml0cm9nZW4gYW5kIHBob3NwaG9ydXMpIHJlZHVjdGlvbiB0YXJnZXRzIGZvciBibG9vbSBjb250cm9sIGluIGFmZmVjdGVkIHdhdGVycy4gSW4gYWRkaXRpb24sIHRoZSBlZmZpY2FjeSBvZiBjdXJyZW50bHkgdXNlZCBtZXRob2RzIHRvIHJlZHVjZSBDeWFub0hBQnMgd2lsbCBuZWVkIHRvIGJlIHJlLWV2YWx1YXRlZCBpbiBsaWdodCBvZiB0aGUgc3luZXJnaXN0aWMgZWZmZWN0cyBvZiBjbGltYXRlIGNoYW5nZSB3aXRoIG51dHJpZW50IGVucmljaG1lbnQuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjEwNzEvbWYxODM5MiIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9jZHIubGliLnVuYy5lZHUvZG93bmxvYWRzLzA1NzQyMjgzNCIsImlzc3VlZERhdGUiOiIyMDE5LTAzLTI1IiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiTWFyaW5lIGFuZCBGcmVzaHdhdGVyIFJlc2VhcmNoIiwiaXNzdWUiOjUsInZvbHVtZSI6NzEsImltcGFjdEZhY3RvciI6IjIuNzAifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjo1NzksImxhc3RQYWdlIjo1Nzl9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiQ1NJUk8gUHVibGlzaGluZyJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MTA0LCJjaXRlZEJ5Q291bnQiOjkxfX0sInF1b3RlIjoiVGhhdCByZWxhdGlvbnNoaXAgaGFzIHNlcnZlZCBhcyB0aGUgZm91bmRhdGlvbiBmb3Igc2V0dGluZyAgbnV0cmllbnQgbG9hZGluZyB0YXJnZXRzIGluIGxha2VzIGFyb3VuZCB0aGUgd29ybGQsIGFuZCBjYXJlZnVsbHkgIGRlc2lnbmVkIGV4cGVyaW1lbnRzIG1heSBlbHVjaWRhdGUgdG8gd2hhdCBleHRlbnQgdGVtcGVyYXR1cmUgIGluY3JlYXNlIGFmZmVjdHMgdGhlIHRocmVzaG9sZCBhdCB3aGljaCB0aGUgZnJlcXVlbmN5IG9mIG9jY3VyICByZW5jZSBvZiBDeWFub0hBQnMgcXVpY2tseSByaXNlcyB3aXRoIGluY3JlYXNpbmcgTiBhbmQgUC4gQ29uY2x1c2lvbnMgUmVjZW50IHN0dWRpZXMgaW5kaWNhdGUgdGhhdCB0aGUgc3luZXJnaXN0aWMgY29tYmluYXRpb24gb2YgYW50aHJvcG9nZW5pYyBudXRyaWVudCBsb2FkaW5nLCByaXNpbmcgdGVtcGVyYXR1cmVzLCBlbmhhbmNlZCAgdmVydGljYWwgc3RyYXRpZmljYXRpb24sIGluY3JlYXNlZCByZXNpZGVuY2UgdGltZSBhbmQgbW9yZSAgZXh0cmVtZSBjbGltYXRpYyBjb25kaXRpb25zIG92ZXJhbGwgd2lsbCBmYXZvdXIgY3lhbm9iYWN0ZXJpYWwgIGRvbWluYW5jZSBpbiBhIHdpZGUgcmFuZ2Ugb2YgYXF1YXRpYyBlY29zeXN0ZW1zLiBUaGUgZXhwYW4gIHNpb24gb2YgQ3lhbm9IQUJzIGhhcyBzZXJpb3VzIGNvbnNlcXVlbmNlcyBmb3IgaHVtYW4gd2F0ZXIgIHN1cHBsaWVzLCBmaXNoZXJpZXMgYW5kIHJlY3JlYXRpb25hbCByZXNvdXJjZXMuIEVjb3N5c3RlbSBzY2FsZSAgcGh5c2ljYWwgY2hlbWljYWwgYmlvdGljIG1pdGlnYXRpb24gc3RyYXRlZ2llcyB3aWxsIGhhdmUgdG8gIGluY29ycG9yYXRlIG51dHJpZW50IChtb3N0IG9mdGVuIGJvdGggTiBhbmQgUCkgaW5wdXQgcmVkdWN0aW9ucy4gIFRoZSBtYWduaXR1ZGVzIG9mIHRoZXNlIHJlZHVjdGlvbnMgd2lsbCBuZWVkIHRvIGJlIHN5c3RlbSAgc3BlY2lmaWMgYW5kIHdpbGwgbGlrZWx5IG5lZWQgdG8gYmUgYWRqdXN0ZWQgd2l0aCBjaGFuZ2luZyBjbGkgIG1hdGljIGNvbmRpdGlvbnMuIFNwZWNpZmljYWxseSwgd2F0ZXIgcXVhbGl0eSBtYW5hZ2VycyB3aWxsIGhhdmUgIHRvIGFjY29tbW9kYXRlIHdhcm1pbmcgYW5kIGdyZWF0ZXIgaHlkcm9sb2dpY2FsIHZhcmlhYmlsaXR5ICBhbmQgZXh0cmVtZW5lc3MgcmVzdWx0aW5nIGZyb20gY2xpbWF0ZSBjaGFuZ2UgaW4gZm9ybXVsYXRpbmcgIHRoZWlyIHN0cmF0ZWdpZXMuIEEga2V5IGxvbmcgdGVybSBjb250cm9sIHdlIGNhbiBleGVydCB0byByZWR1Y2UgIHRoZSByYXRlIGFuZCBleHRlbnQgb2YgZ2xvYmFsIHdhcm1pbmcgaXMgY3VyYmluZyBncmVlbmhvdXNlIGdhcyAgZW1pc3Npb25zLiBXaXRob3V0IHRoaXMgZXNzZW50aWFsIHN0ZXAsIGZ1dHVyZSB3YXJtaW5nIHRyZW5kcywgIGh5ZHJvbG9naWNhbCBleHRyZW1lcyAocmVjb3JkIHJhaW5mYWxscyBhbmQgZHJvdWdodHMpIGFuZCB0aGVpciAgZWZmZWN0cyBvbiBhcXVhdGljIGVjb3N5c3RlbXMgd2lsbCBjb250aW51ZSB0byBwcm9tb3RlIG9wcG9yICB0dW5pc3RpYyBDeWFub0hBQnMuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjE0LjE0OTIzMzgxODA1NDIsImxlZnQiOjEzLjEyODU5NTM1MjE3Mjg1Miwid2lkdGgiOjM3LjA4MjE3MjM5Mzc5ODgzLCJoZWlnaHQiOjEuNTE2NjAyMjc3NzU1NzM3M319LHsicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjoxNS40NTE1MDg1MjIwMzM2OTEsImxlZnQiOjEzLjEyODU0NDgwNzQzNDA4Miwid2lkdGgiOjM3LjA4NDgzMTIzNzc5Mjk3LCJoZWlnaHQiOjEuNTE2NjAzMjMxNDMwMDUzN319LHsicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjoxNi43NTM3ODQxNzk2ODc1LCJsZWZ0IjoxMy4xMjg1NDI5MDAwODU0NSwid2lkdGgiOjM3LjA4MzIwOTk5MTQ1NTA4LCJoZWlnaHQiOjEuNTE2NjA0MTg1MTA0MzcwMX19LHsicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjoxOC4wNTYwNTMxNjE2MjEwOTQsImxlZnQiOjEzLjEyODU0NDgwNzQzNDA4Miwid2lkdGgiOjM2LjYxNTA3NDE1NzcxNDg0NCwiaGVpZ2h0IjoxLjUxNjYwNDE4NTEwNDM3MDF9fSx7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6MTkuNDY4Mzc4MDY3MDE2NiwibGVmdCI6MTMuMTI4NTQ0ODA3NDM0MDgyLCJ3aWR0aCI6MzQuMjkwNTMxMTU4NDQ3MjY2LCJoZWlnaHQiOjEuMjQ1MDExMjEwNDQxNTg5NH19LHsicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjoyMi4yMjYxNDI4ODMzMDA3OCwibGVmdCI6MTMuMTI4NTUwNTI5NDc5OTgsIndpZHRoIjo3LjY0OTg3MzczMzUyMDUwOCwiaGVpZ2h0IjoxLjEzNTQ4MzE0NTcxMzgwNjJ9fSx7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6MjMuNzM0ODQ5OTI5ODA5NTcsImxlZnQiOjEzLjEyODUzNTI3MDY5MDkxOCwid2lkdGgiOjM3LjA4OTI4NjgwNDE5OTIyLCJoZWlnaHQiOjI2LjA4ODY0MDIxMzAxMjY5NX19XX0sImNodW5rSWQiOiJhZjA2MDcwOS1mNTcwLTQ4MmQtOTcxYS05YTRkZDhhODUwNWQiLCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI1ODkifX1d. The expansion of CyanoHABs has serious consequences for human water supplies, fisheries and recreational resources (Paerl et al., 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhhbnMgVy4iLCJmYW1pbHkiOiJQYWVybCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLLkUuIiwiZmFtaWx5IjoiSGF2ZW5zIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik4uUy4iLCJmYW1pbHkiOiJIYWxsIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlQuRy4iLCJmYW1pbHkiOiJPdHRlbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNLiIsImZhbWlseSI6IlpodSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJIYWlodWkiLCJmYW1pbHkiOiJYdSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJHLiIsImZhbWlseSI6IlpodSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJCLiIsImZhbWlseSI6IlFpbiJ9fV0sImlkcyI6eyJpc3NuIjpbXSwiaXNibiI6W10sImRvaSI6IjEwLjE3NjE1LzRua2Qtems5MyIsIm9wZW5hbGV4IjoiVzQzMDcwNjI0MjkiLCJvcGVuQWNjZXNzSWQiOiIxNjQ2OTkwNTc1NTk3MDA5OTQxNSJ9LCJ0aXRsZSI6Ik1pdGlnYXRpbmcgYSBnbG9iYWwgZXhwYW5zaW9uIG9mIHRveGljIGN5YW5vYmFjdGVyaWFsIGJsb29tczogQ29uZm91bmRpbmcgZWZmZWN0cyBhbmQgY2hhbGxlbmdlcyBwb3NlZCBieSBjbGltYXRlIGNoYW5nZSIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xNzYxNS80bmtkLXprOTMiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vY2RyLmxpYi51bmMuZWR1L2Rvd25sb2Fkcy8wNTc0MjI4MzQiLCJpc3N1ZWREYXRlIjoiMjAyMC0wMS0wMSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDEiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkNhcm9saW5hIERpZ2l0YWwgUmVwb3NpdG9yeSAoVW5pdmVyc2l0eSBvZiBOb3J0aCBDYXJvbGluYSBhdCBDaGFwZWwgSGlsbCkifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiVW5pdmVyc2l0eSBvZiBOb3J0aCBDYXJvbGluYSBhdCBDaGFwZWwgSGlsbCJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MCwiY2l0ZWRCeUNvdW50IjowfX0sInF1b3RlIjoiVGhhdCByZWxhdGlvbnNoaXAgaGFzIHNlcnZlZCBhcyB0aGUgZm91bmRhdGlvbiBmb3Igc2V0dGluZyAgbnV0cmllbnQgbG9hZGluZyB0YXJnZXRzIGluIGxha2VzIGFyb3VuZCB0aGUgd29ybGQsIGFuZCBjYXJlZnVsbHkgIGRlc2lnbmVkIGV4cGVyaW1lbnRzIG1heSBlbHVjaWRhdGUgdG8gd2hhdCBleHRlbnQgdGVtcGVyYXR1cmUgIGluY3JlYXNlIGFmZmVjdHMgdGhlIHRocmVzaG9sZCBhdCB3aGljaCB0aGUgZnJlcXVlbmN5IG9mIG9jY3VyICByZW5jZSBvZiBDeWFub0hBQnMgcXVpY2tseSByaXNlcyB3aXRoIGluY3JlYXNpbmcgTiBhbmQgUC4gQ29uY2x1c2lvbnMgUmVjZW50IHN0dWRpZXMgaW5kaWNhdGUgdGhhdCB0aGUgc3luZXJnaXN0aWMgY29tYmluYXRpb24gb2YgYW50aHJvcG9nZW5pYyBudXRyaWVudCBsb2FkaW5nLCByaXNpbmcgdGVtcGVyYXR1cmVzLCBlbmhhbmNlZCAgdmVydGljYWwgc3RyYXRpZmljYXRpb24sIGluY3JlYXNlZCByZXNpZGVuY2UgdGltZSBhbmQgbW9yZSAgZXh0cmVtZSBjbGltYXRpYyBjb25kaXRpb25zIG92ZXJhbGwgd2lsbCBmYXZvdXIgY3lhbm9iYWN0ZXJpYWwgIGRvbWluYW5jZSBpbiBhIHdpZGUgcmFuZ2Ugb2YgYXF1YXRpYyBlY29zeXN0ZW1zLiBUaGUgZXhwYW4gIHNpb24gb2YgQ3lhbm9IQUJzIGhhcyBzZXJpb3VzIGNvbnNlcXVlbmNlcyBmb3IgaHVtYW4gd2F0ZXIgIHN1cHBsaWVzLCBmaXNoZXJpZXMgYW5kIHJlY3JlYXRpb25hbCByZXNvdXJjZXMuIEVjb3N5c3RlbSBzY2FsZSAgcGh5c2ljYWwgY2hlbWljYWwgYmlvdGljIG1pdGlnYXRpb24gc3RyYXRlZ2llcyB3aWxsIGhhdmUgdG8gIGluY29ycG9yYXRlIG51dHJpZW50IChtb3N0IG9mdGVuIGJvdGggTiBhbmQgUCkgaW5wdXQgcmVkdWN0aW9ucy4gIFRoZSBtYWduaXR1ZGVzIG9mIHRoZXNlIHJlZHVjdGlvbnMgd2lsbCBuZWVkIHRvIGJlIHN5c3RlbSAgc3BlY2lmaWMgYW5kIHdpbGwgbGlrZWx5IG5lZWQgdG8gYmUgYWRqdXN0ZWQgd2l0aCBjaGFuZ2luZyBjbGkgIG1hdGljIGNvbmRpdGlvbnMuIFNwZWNpZmljYWxseSwgd2F0ZXIgcXVhbGl0eSBtYW5hZ2VycyB3aWxsIGhhdmUgIHRvIGFjY29tbW9kYXRlIHdhcm1pbmcgYW5kIGdyZWF0ZXIgaHlkcm9sb2dpY2FsIHZhcmlhYmlsaXR5ICBhbmQgZXh0cmVtZW5lc3MgcmVzdWx0aW5nIGZyb20gY2xpbWF0ZSBjaGFuZ2UgaW4gZm9ybXVsYXRpbmcgIHRoZWlyIHN0cmF0ZWdpZXMuIEEga2V5IGxvbmcgdGVybSBjb250cm9sIHdlIGNhbiBleGVydCB0byByZWR1Y2UgIHRoZSByYXRlIGFuZCBleHRlbnQgb2YgZ2xvYmFsIHdhcm1pbmcgaXMgY3VyYmluZyBncmVlbmhvdXNlIGdhcyAgZW1pc3Npb25zLiBXaXRob3V0IHRoaXMgZXNzZW50aWFsIHN0ZXAsIGZ1dHVyZSB3YXJtaW5nIHRyZW5kcywgIGh5ZHJvbG9naWNhbCBleHRyZW1lcyAocmVjb3JkIHJhaW5mYWxscyBhbmQgZHJvdWdodHMpIGFuZCB0aGVpciAgZWZmZWN0cyBvbiBhcXVhdGljIGVjb3N5c3RlbXMgd2lsbCBjb250aW51ZSB0byBwcm9tb3RlIG9wcG9yICB0dW5pc3RpYyBDeWFub0hBQnMuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjE0LjE0OTIzMzgxODA1NDIsImxlZnQiOjEzLjEyODU5NTM1MjE3Mjg1Miwid2lkdGgiOjM3LjA4MjE3MjM5Mzc5ODgzLCJoZWlnaHQiOjEuNTE2NjAyMjc3NzU1NzM3M319LHsicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjoxNS40NTE1MDg1MjIwMzM2OTEsImxlZnQiOjEzLjEyODU0NDgwNzQzNDA4Miwid2lkdGgiOjM3LjA4NDgzMTIzNzc5Mjk3LCJoZWlnaHQiOjEuNTE2NjAzMjMxNDMwMDUzN319LHsicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjoxNi43NTM3ODQxNzk2ODc1LCJsZWZ0IjoxMy4xMjg1NDI5MDAwODU0NSwid2lkdGgiOjM3LjA4MzIwOTk5MTQ1NTA4LCJoZWlnaHQiOjEuNTE2NjA0MTg1MTA0MzcwMX19LHsicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjoxOC4wNTYwNTMxNjE2MjEwOTQsImxlZnQiOjEzLjEyODU0NDgwNzQzNDA4Miwid2lkdGgiOjM2LjYxNTA3NDE1NzcxNDg0NCwiaGVpZ2h0IjoxLjUxNjYwNDE4NTEwNDM3MDF9fSx7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6MTkuNDY4Mzc4MDY3MDE2NiwibGVmdCI6MTMuMTI4NTQ0ODA3NDM0MDgyLCJ3aWR0aCI6MzQuMjkwNTMxMTU4NDQ3MjY2LCJoZWlnaHQiOjEuMjQ1MDExMjEwNDQxNTg5NH19LHsicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjoyMi4yMjYxNDI4ODMzMDA3OCwibGVmdCI6MTMuMTI4NTUwNTI5NDc5OTgsIndpZHRoIjo3LjY0OTg3MzczMzUyMDUwOCwiaGVpZ2h0IjoxLjEzNTQ4MzE0NTcxMzgwNjJ9fSx7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6MjMuNzM0ODQ5OTI5ODA5NTcsImxlZnQiOjEzLjEyODUzNTI3MDY5MDkxOCwid2lkdGgiOjM3LjA4OTI4NjgwNDE5OTIyLCJoZWlnaHQiOjI2LjA4ODY0MDIxMzAxMjY5NX19XX0sImNodW5rSWQiOiI2NTEwYjg4YS00MDliLTQ1NmEtODM3MC04YjU2NTA3OGExZDciLCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIxMSJ9fV0%3D. Specifically, the toxic secondary metabolites generated by these proliferations compromise the safety of drinking water and pose severe risks to human health (Shahnazaryan et al., 2023; Wang et al., 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkdheWFuZSIsImZhbWlseSI6IlNoYWhuYXphcnlhbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNYXJ0aW4iLCJmYW1pbHkiOiJTY2h1bHR6ZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLYXJzdGVuIiwiZmFtaWx5IjoiUmlua2UifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQmFyZHVraCIsImZhbWlseSI6IkdhYnJpZWx5YW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIxMTI5LTU3NjciLCIxNzIzLTg2MzMiXSwiaXNibiI6W10sImRvaSI6IjEwLjQwODEvamxpbW5vbC4yMDIyLjIxNjgiLCJvcGVuYWxleCI6Ilc0Mzg4OTc3MDMyIiwib3BlbkFjY2Vzc0lkIjoiMTY3MDU4NTU2NjE3MzE4MjkyOTgifSwidGl0bGUiOiJMYWtlIFNldmFuLiBQYXN0LCBwcmVzZW50LCBhbmQgZnV0dXJlIHN0YXRlIG9mIGEgdW5pcXVlIGFscGluZSBsYWtlIiwiYWJzdHJhY3QiOiJMYWtlIFNldmFuLCBhIGxhcmdlLCBkZWVwLCBhbHBpbmUgbGFrZSBpbiB0aGUgTGVzc2VyIENhdWNhc3VzIGlzIHRoZSBmb2N1cyBvZiB0aGlzIFNwZWNpYWwgSXNzdWUgb2YgdGhlIEpvdXJuYWwgb2YgTGltbm9sb2d5LiBJdCB3YXMgYW4gb3V0c3RhbmRpbmcgZWNvc3lzdGVtIDEwMCB5ZWFycyBhZ28gY2hhcmFjdGVyaXNlZCBieSBleGNlbGxlbnQgd2F0ZXIgcXVhbGl0eSwgcmljaCBiaW9kaXZlcnNpdHkgd2l0aCBhIGhpZ2ggbGV2ZWwgb2YgZW5kZW1pc20sIHdpZGUtcmFuZ2luZyBiZWRzIG9mIG1hY3JvcGh5dGVzIGFsb25nIHRoZSBzaG9yZXMgYW5kIGEgcHJvZHVjdGl2ZSBhbmQgc3VzdGFpbmFibGUgZmlzaCBwcm9kdWN0aW9uLiBEdWUgdG8gaXRzIGJlYXV0eSwgbmF0dXJhbCBoaXN0b3J5LCBhbmQgY29udHJpYnV0aW9ucyB0byBzb2NpYWwgYW5kIGVjb25vbWljIHdlbGZhcmUgaXQgaXMgYWxzbyBhIGN1bHR1cmFsIGhlcml0YWdlIGZvciB0aGUgQXJtZW5pYW4gTmF0aW9uIGluY2x1ZGluZyBpdHMgbGFyZ2UgZGlhc3BvcmEuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjQwODEvamxpbW5vbC4yMDIyLjIxNjgiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vd3d3LmpsaW1ub2wuaXQvaW5kZXgucGhwL2psaW1ub2wvYXJ0aWNsZS9kb3dubG9hZC8yMTY4LzIwMDciLCJpc3N1ZWREYXRlIjoiMjAyMy0xMS0yMyIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkpvdXJuYWwgb2YgTGltbm9sb2d5Iiwidm9sdW1lIjo4MSwiaW1wYWN0RmFjdG9yIjoiMi4wMSJ9LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJQQUdFUHJlc3MgKEl0YWx5KSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MzUsImNpdGVkQnlDb3VudCI6Mn19LCJxdW90ZSI6IkNsaW1hdGUgd2FybWluZyBjYXVzZXMgY2hhbmdlcyBpbiB0aGUgdGhlcm1hbCByZWdpbWUgb2YgbGFrZXMsIGkuZS4sIHRoZSB0eXBpY2FsIHRlbXBlcmF0dXJlIHN0cnVjdHVyZXMgKFdvb2x3YXkgYW5kIE1lci1jaGFudCwgMjAxOSkgYW5kIHRoZSB0aW1pbmcgb2YgbWl4aW5nIGFuZCBzdHJhdGlmaWNhdGlvbiAoS3JhZW1lciBldCBhbC4sIDIwMjEpLCB0aGUgZHVyYXRpb24gb2Ygc3VtbWVyIHN0cmF0aWZpY2F0aW9uICBhbmQgaWNlIGNvdmVyIChSb2dvcmEgZXQgYWwuLCAyMDE4OyBTaGFybWEgZXQgYWwuLCAyMDE5KSBhbmQsIGV2ZW50dWFsbHksIGl0IGlzIGFjY2VsZXJhdGluZyBpbnRlcm5hbCBudXRyaWVudCBsb2FkaW5nIChOb3J0aCBldCBhbC4sIDIwMTQpIGFuZCBpbXBhY3RpbmcgdGhlIGVudGlyZSBsYWtlIGVjb3N5c3RlbSAgKFNjaHdlZmVsIGV0IGFsLiwgMjAxOTsgV29vbHdheSBldCBhbC4sIDIwMjIpLiBSZWNlbnQgc3R1ZGllcyB3b3JsZHdpZGUgZG9jdW1lbnRlZCB1bmVxdWl2b2NhbGx5IHRoYXQgdGhlIHR5cGljYWwgc3RyZXNzb3JzIG9mIGxha2VzIGludGVyYWN0LCBwb3RlbnRpYWxseSBsZWFkLWluZyB0byBlbmZvcmNlZCBuZWdhdGl2ZSBjb25zZXF1ZW5jZXMuIFRoZXJlIGlzIGNvbnNpZGVyLWFibGUgZXZpZGVuY2UgdGhhdCBjbGltYXRlIGNoYW5nZSBpcyBleGFjZXJiYXRpbmcgIGV1dHJvcGhpY2F0aW9uIGFuZCBIQUJzIChQZWFybCBhbmQgSHVpc21hbiwgMjAwODsgV29vbC13YXkgZXQgYWwuLCAyMDIwOyBNZWVyaG9mZiBldCBhbC4sIDIwMjIpIGFuZCBvdGhlciBlZmZlY3RzICBrbm93biBmcm9tIGV1dHJvcGhpY2F0aW9uIGxpa2UgaHlwb3hpYSBhbmQgYW5veGlhIG9mIHRoZSAgaHlwb2xpbW5pb24gb2Ygc3RyYXRpZmllZCBsYWtlcyAoRnVrdXNoaW1hIGV0IGFsLiwgMjAxNzsgIFJvZ29yYSBldCBhbC4sIDIwMTg7IEphbmUgZXQgYWwuLCAyMDIxKS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo1NC4yNTAyMzI2OTY1MzMyLCJsZWZ0Ijo3LjE0MzM5MzAzOTcwMzM2OSwid2lkdGgiOjM4LjQ5OTI2NzU3ODEyNSwiaGVpZ2h0IjoxLjM5NTI2MzMxNDI0NzEzMTN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjoyNS41MjI3OTI4MTYxNjIxMSwibGVmdCI6NDkuOTk5MTQ1NTA3ODEyNSwid2lkdGgiOjM4LjUwNjI5NDI1MDQ4ODI4LCJoZWlnaHQiOjEyLjEyOTc3NTA0NzMwMjI0Nn19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjM5LjMyMjY4NTI0MTY5OTIyLCJsZWZ0Ijo1Mi4zODEyMzcwMzAwMjkzLCJ3aWR0aCI6MzYuMTI0NDk2NDU5OTYwOTQsImhlaWdodCI6MS4zOTUyNjMzMTQyNDcxMzEzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NDAuODU2MDE0MjUxNzA4OTg0LCJsZWZ0Ijo1MC4wMDA4MjM5NzQ2MDkzNzUsIndpZHRoIjozOC4wODQ0OTU1NDQ0MzM1OTQsImhlaWdodCI6MS4zOTUyNjMzMTQyNDcxMzEzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NDIuMzg5MzM1NjMyMzI0MjIsImxlZnQiOjUwLjAwMDgyMzk3NDYwOTM3NSwid2lkdGgiOjM4LjA0ODU1NzI4MTQ5NDE0LCJoZWlnaHQiOjEuMzk1MjYzMzE0MjQ3MTMxM319LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjQzLjkyMjY2MDgyNzYzNjcyLCJsZWZ0Ijo1MC4wMDA4MjM5NzQ2MDkzNzUsIndpZHRoIjozOC41MDU1MTIyMzc1NDg4MywiaGVpZ2h0IjoxLjM5NTI2MzMxNDI0NzEzMTN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo0NS40NTU5ODYwMjI5NDkyMiwibGVmdCI6NTAuMDAwODIzOTc0NjA5Mzc1LCJ3aWR0aCI6MzguMDgwNzgwMDI5Mjk2ODc1LCJoZWlnaHQiOjEuMzk1MjYzMzE0MjQ3MTMxM319LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjQ2Ljk5MDU2NjI1MzY2MjExLCJsZWZ0Ijo1MC4wMDA4MjM5NzQ2MDkzNzUsIndpZHRoIjozOC40OTk5NDY1OTQyMzgyOCwiaGVpZ2h0IjoxLjM5NTI2MzMxNDI0NzEzMTN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo0OC41MjI2Mjg3ODQxNzk2OSwibGVmdCI6NTAuMDAwODIzOTc0NjA5Mzc1LCJ3aWR0aCI6MzguNTAzMTEyNzkyOTY4NzUsImhlaWdodCI6MS4zOTUyNjMzMTQyNDcxMzEzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NTAuMDU3MjEyODI5NTg5ODQ0LCJsZWZ0Ijo1MC4wMDA4MjM5NzQ2MDkzNzUsIndpZHRoIjozOC40OTg5Mzk1MTQxNjAxNTYsImhlaWdodCI6MS4zOTUyNjMzMTQyNDcxMzEzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NTEuNTkwNTM4MDI0OTAyMzQ0LCJsZWZ0Ijo1MC4wMDA4Mjc3ODkzMDY2NCwid2lkdGgiOjM4LjUwMTA3OTU1OTMyNjE3LCJoZWlnaHQiOjEuMzk1MjYzMzE0MjQ3MTMxM319XX0sImNodW5rSWQiOiI1ZWI0NGNlMS1jZDI5LTRmMDgtODIzOS04NzBmYjBiZTQ4MzUiLCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIxIn19LHsib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ilhpd2VuIiwiZmFtaWx5IjoiV2FuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLdW4iLCJmYW1pbHkiOiJTaGkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiWXVubGluIiwiZmFtaWx5IjoiWmhhbmcifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQm9xaWFuZyIsImZhbWlseSI6IlFpbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJZaWJvIiwiZmFtaWx5IjoiWmhhbmcifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiV2VpamlhIiwiZmFtaWx5IjoiV2FuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJSLiBJZXN0eW4iLCJmYW1pbHkiOiJXb29sd2F5In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNoaWxvbmciLCJmYW1pbHkiOiJQaWFvIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkVyaWsiLCJmYW1pbHkiOiJKZXBwZXNlbiJ9fV0sImlkcyI6eyJpc3NuIjpbXSwiaXNibiI6W10sImRvaSI6IjEwLjIxMjAzL3JzLjMucnMtMjQ0MjE0Mi92MSIsIm9wZW5hbGV4IjoiVzQzMTcwMzg4MjciLCJvcGVuQWNjZXNzSWQiOiIxOTg1NTc4NDU1NjIyNTM2MDQ0In0sInRpdGxlIjoiTGFrZSB3YXJtaW5nIGFuZCBpbmNyZWFzZWQgaGVhdHdhdmVzIHJldmVhbGVkIGJ5IGEgbm92ZWwgZGF0YS1kcml2ZW4gbW9kZWxpbmcgYXBwcm9hY2giLCJhYnN0cmFjdCI6Ijx0aXRsZT5BYnN0cmFjdDwvdGl0bGU%2BIENsaW1hdGUgY2hhbmdlIGNvdWxkIHNlcmlvdXNseSB0aHJlYXRlbiBnbG9iYWwgbGFrZSBlY29zeXN0ZW1zIGJ5IHdhcm1pbmcgbGFrZSBzdXJmYWNlIHdhdGVyIGFuZCBpbmNyZWFzaW5nIHRoZSBvY2N1cnJlbmNlIG9mIGxha2UgaGVhdHdhdmVzLiBZZXQsIHRoZXJlIGFyZSBncmVhdCB1bmNlcnRhaW50aWVzIGluIHF1YW50aWZ5aW5nIGxha2UgdGVtcGVyYXR1cmUgY2hhbmdlcyBnbG9iYWxseSBkdWUgdG8gbGFjayBvZiBhY2N1cmF0ZSBsYXJnZS1zY2FsZSBtb2RlbCBzaW11bGF0aW9ucy4gSGVyZSwgd2UgaW50ZWdyYXRlZCBzYXRlbGxpdGUgb2JzZXJ2YXRpb25zIGFuZCBhIG51bWVyaWNhbCBtb2RlbCB0byBpbXByb3ZlIGxha2UgdGVtcGVyYXR1cmUgbW9kZWxsaW5nIGFuZCB0byBleHBsb3JlIHRoZSBtdWx0aWZhY2V0ZWQgY2hhcmFjdGVyaXN0aWNzIG9mIHRyZW5kcyBpbiBzdXJmYWNlIHRlbXBlcmF0dXJlcyBhbmQgbGFrZSBoZWF0d2F2ZSBvY2N1cnJlbmNlIGluIENoaW5lc2UgbGFrZXMgZnJvbSAxOTgwIHRvIDIxMDAuIE91ciBtb2RlbC1kYXRhIGludGVncmF0aW9uIGFwcHJvYWNoIHJldmVhbGVkIHRoYXQgdGhlIGxha2Ugc3VyZmFjZSB3YXRlcnMgaGF2ZSB3YXJtZWQgYXQgYSByYXRlIG9mIDAuMTEgwrBDIGRlY2FkZS0xIGR1cmluZyB0aGUgcGVyaW9kIDE5ODAtMjAyMSwgYmVpbmcgb25seSBoYWxmIG9mIHRoZSBwdXJlIG1vZGVsLWJhc2VkIGVzdGltYXRlLiBNb3Jlb3Zlciwgb3VyIGFuYWx5c2lzIHN1Z2dlc3RlZCB0aGF0IGFuIGFzeW1tZXRyaWMgc2Vhc29uYWwgd2FybWluZyByYXRlIGhhcyBsZWQgdG8gYSByZWR1Y2VkIHRlbXBlcmF0dXJlIHNlYXNvbmFsaXR5IGluIGVhc3Rlcm4gcGxhaW4gbGFrZXMgYnV0IGFuIGFtcGxpZmllZCBvbmUgaW4gYWxwaW5lIGxha2VzLiBUaGUgZHVyYXRpb25zIG9mIGxha2UgaGVhdHdhdmVzIGhhdmUgYWxzbyBpbmNyZWFzZWQgYXQgYSByYXRlIG9mIDcuNyBkYXlzIGRlY2FkZS0xLiBVbmRlciB0aGUgaGlnaC1ncmVlbmhvdXNlLWdhcy1lbWlzc2lvbiBzY2VuYXJpbywgbGFrZSBzdXJmYWNlIHRlbXBlcmF0dXJlIGFuZCBsYWtlIGhlYXR3YXZlIGR1cmF0aW9uIHdlcmUgcHJvamVjdGVkIHRvIGluY3JlYXNlIGJ5IDIuMiDCsEMgYW5kIDE5NyBkYXlzIGF0IHRoZSBlbmQgb2YgMjFzdCBjZW50dXJ5LCByZXNwZWN0aXZlbHkuIFN1Y2ggZHJhc3RpYyBjaGFuZ2VzIHdvdWxkIHdvcnNlbiB0aGUgZW52aXJvbm1lbnRhbCBjb25kaXRpb25zIG9mIGxha2VzIHN1YmplY3RlZCB0byBoaWdoIGFuZCBpbmNyZWFzaW5nIGFudGhyb3BvZ2VuaWMgcHJlc3N1cmVzLCBwb3NpbmcgZ3JlYXQgdGhyZWF0cyB0byBhcXVhdGljIGJpb2RpdmVyc2l0eSBhbmQgaHVtYW4gaGVhbHRoLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4yMTIwMy9ycy4zLnJzLTI0NDIxNDIvdjEiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vd3d3LnJlc2VhcmNoc3F1YXJlLmNvbS9hcnRpY2xlL3JzLTI0NDIxNDIvbGF0ZXN0LnBkZiIsImlzc3VlZERhdGUiOiIyMDIzLTAxLTE3IiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiUmVzZWFyY2ggU3F1YXJlIChSZXNlYXJjaCBTcXVhcmUpIiwiaW1wYWN0RmFjdG9yIjoiMC40MCJ9LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJSZXNlYXJjaCBTcXVhcmUgKFVuaXRlZCBTdGF0ZXMpIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo1NywiY2l0ZWRCeUNvdW50IjozfX0sInF1b3RlIjoiSWYgdGhlIGJvdHRvbSB3YXRlciB0ZW1wZXJhdHVyZSBhbHNvIGluY3JlYXNlcywgbWluZXJhbGl6YXRpb24gb2Ygb3JnYW5pYyBtYXR0ZXIgd2lsbCBiZSBhY2NlbGVyYXRlZCwgd2hpY2ggd2lsbCBsZWFkIHRvIGhpZ2hlciBlbWlzc2lvbiBvZiBjYXJib24gZGlveGlkZSBhbmQgbWV0aGFuZTQ2LDQ3LCByZWluZm9yY2luZyBjbGltYXRlIHdhcm1pbmcgZHVlIHRvIGdyZWVuaG91c2UgZ2FzZXMuIFRoaXMgcHJvY2VzcyB3aWxsIHNwZWVkIHVwIHRoZSByZWN5Y2xpbmcgb2YgbnV0cmllbnRzIGFuZCBmYXZvciBwaHl0b3BsYW5rdG9uIGdyb3d0aCwgc3VnZ2VzdGluZyBwb3NpdGl2ZSBmZWVkYmFjayBiZXR3ZWVuIGNsaW1hdGUgY2hhbmdlIGFuZCBldXRyb3BoaWNhdGlvbjQ4LiBUb2dldGhlciB3aXRoIHRoZSBoZWF2eSBleHRlcm5hbCBudXRyaWVudCBsb2FkaW5nIGZyb20gZG9tZXN0aWMsIGFncmljdWx0dXJhbCwgYW5kIGluZHVzdHJpYWwgd2FzdGUsIHRoZSBncm93dGggb2YgcGh5dG9wbGFua3RvbiBpcyBwcm9tb3RlZCBieSBob3Qgd2F0ZXIgdW5kZXIgbnV0cmllbnQtcmljaCBjb25kaXRpb25zNDkuIEJlc2lkZXMgdGhhdCwgc3Ryb25nZXIgc3RyYXRp77%2B9Y2F0aW9uIHdpbGwgcGVybWl0IHBoeXRvcGxhbmt0b24gdG8gY2lyY3VsYXRlIGluIHRoZSBldXBob3RpYyB6b25lNTAuIEluY3JlYXNpbmcgYWxnYWUgYmxvb21zIGhhdmUgYmVlbiByZXBvcnRlZCB3b3JsZHdpZGUsIGVzcGVjaWFsbHkgaW4gcmVnaW9ucyB0aGF0IGRlcGVuZCBoaWdobHkgb24gZmVydGlsaXplcnM1MS4gSW4gQ2hpbmEsIHRoZSBvY2N1cnJlbmNlIG9mIGFsZ2FlIGJsb29tcyBpbiB0aGUgMjAxMHMgaGFzIGluY3JlYXNlZCBuZWFybHkgbmluZSB0aW1lcyByZWxhdGl2ZSB0byB0aGUgMTk4MHMtMTk5MHM1MSwgYW5kIG1hbnkgYmxvb21zIGhhdmUgYmVlbiByZXBvcnRlZCByZWNlbnRseSBpbiBuYXR1cmFsIGxha2VzIGFuZCByZXNlcnZvaXJzIChTdXBwbGVtZW50YXJ5IEZpZy4gNykuIEZ1cnRoZXJtb3JlLCB0aGUgd2FybWVyIHN1cmZhY2Ugd2F0ZXIgYW5kIHN0cm9uZ2VyIHN0cmF0ae%2B%2FvWNhdGlvbiB3aWxsIGZhdm9yIHRoZSBkb21pbmFuY2Ugb2YgY3lhbm9iYWN0ZXJpYSBpbiBsYWtlcyBnaXZlbiBpdHMgaGlnaGVyIG9wdGltYWwgZ3Jvd3RoIHRlbXBlcmF0dXJlIGFuZCBhYmlsaXR5IHRvIGFkanVzdCBidW95YW5jeTQ5LCBsZWFkaW5nIHRvIGhpZ2hlciBmcmVxdWVuY3kgb2YgaGFybWZ1bCBhbGdhZSBibG9vbXM0Ny4gVGhlIHRveGljIHN1YnN0YW5jZXMgcHJvZHVjZWQgYnkgY3lhbm9iYWN0ZXJpYSB3aWxsIHRocmVhdGVuIHRoZSBzZWN1cml0eSBvZiBkcmlua2luZyB3YXRlciBhbmQgaHVtYW4gaGVhbHRoNiw1Mi4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo0NS4xMjY5NjA3NTQzOTQ1MywibGVmdCI6NC41NzUxNjI4ODc1NzMyNDIsIndpZHRoIjo5MC45NDEyMDAyNTYzNDc2NiwiaGVpZ2h0IjoxLjc3NzE2MzM4NjM0NDkwOTd9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo0Ny40MDE3MjE5NTQzNDU3LCJsZWZ0Ijo0LjU3NTE2Mjg4NzU3MzI0Miwid2lkdGgiOjkwLjIwNTI0NTk3MTY3OTY5LCJoZWlnaHQiOjEuNzc3MTYzMzg2MzQ0OTA5N319LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjQ5LjY0OTExNjUxNjExMzI4LCJsZWZ0Ijo0LjU3NTE2Mjg4NzU3MzI0Miwid2lkdGgiOjkxLjA0MzczMTY4OTQ1MzEyLCJoZWlnaHQiOjQuMjY4ODY5ODc2ODYxNTcyfX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NTQuMzg4MjE3OTI2MDI1MzksImxlZnQiOjQuNTc1MTYyODg3NTczMjQyLCJ3aWR0aCI6NDguNDc2MjI2ODA2NjQwNjI1LCJoZWlnaHQiOjEuOTk0MTA4MDgwODYzOTUyNn19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjU4LjM5NjQzNDc4MzkzNTU1LCJsZWZ0Ijo0LjU3NTE2Mjg4NzU3MzI0Miwid2lkdGgiOjg3Ljg4MjM4NTI1MzkwNjI1LCJoZWlnaHQiOjEuNzc3MTYzMzg2MzQ0OTA5N319LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjYwLjY0MzgyOTM0NTcwMzEyNSwibGVmdCI6NC41NzUxNjI4ODc1NzMyNDIsIndpZHRoIjo5MC40NTA1NjE1MjM0Mzc1LCJoZWlnaHQiOjEuOTk0MTA4MDgwODYzOTUyNn19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjYzLjEwODE2OTU1NTY2NDA2LCJsZWZ0Ijo0LjU3NTE2Mjg4NzU3MzI0Miwid2lkdGgiOjg5Ljk2NzU5Nzk2MTQyNTc4LCJoZWlnaHQiOjEuOTk0MTA4MDgwODYzOTUyNn19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjY1LjU3MjQ0ODczMDQ2ODc1LCJsZWZ0Ijo0LjU3NTE2Mjg4NzU3MzI0Miwid2lkdGgiOjkwLjU4MDg4Njg0MDgyMDMxLCJoZWlnaHQiOjEuOTk0MTY5NzEyMDY2NjUwNH19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjY4LjAzNjc4ODk0MDQyOTY5LCJsZWZ0Ijo0LjU3NTE2Mjg4NzU3MzI0Miwid2lkdGgiOjg5LjE5MDQzNzMxNjg5NDUzLCJoZWlnaHQiOjYuNTQzNjkzMDY1NjQzMzEwNX19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjc1LjA1MDY1MTU1MDI5Mjk3LCJsZWZ0Ijo0LjU3NTE2Mjg4NzU3MzI0Miwid2lkdGgiOjkwLjM0NzA2ODc4NjYyMTEsImhlaWdodCI6MS45OTQxNjk3MTIwNjY2NTA0fX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NzcuNTE0OTkxNzYwMjUzOSwibGVmdCI6NC41NzUxNjI4ODc1NzMyNDIsIndpZHRoIjo5MC4yMjI0ODg0MDMzMjAzMSwiaGVpZ2h0IjoxLjk5NDE2OTcxMjA2NjY1MDR9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo3OS45NzkzMzk1OTk2MDkzOCwibGVmdCI6NC41NzUxNjI4ODc1NzMyNDIsIndpZHRoIjo4NS45ODIxMzE5NTgwMDc4MSwiaGVpZ2h0IjoxLjk5NDE2OTcxMjA2NjY1MDR9fV19LCJjaHVua0lkIjoiODQyMDIzNWMtMjQ3NS00MDJlLWJlZTUtOWRkYTk2ZTRkMDVhIiwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiOCJ9fV0%3D.

7. Ecological Consequences: Oxygen Depletion and Aquatic "Dead Zones"

Harmful algal blooms, dominated by cyanobacteria such as Microcystis and Dolichospermum, profoundly disrupt lake oxygen dynamics, initiating a cascade of ecological devastation. During peak bloom phases, dense surface scums generated by these buoyant cyanobacteria intercept sunlight, fueling intense photosynthesis that supersaturates surface waters with dissolved oxygen during daylight(Swann et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1pY2FoIE0uIiwiZmFtaWx5IjoiU3dhbm4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQWxpY2lhIiwiZmFtaWx5IjoiQ29ydMOpcyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBbGV4YW5kZXIgTC4iLCJmYW1pbHkiOiJGb3JyZXN0In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5pY2hvbGFzIFQuIiwiZmFtaWx5IjoiRnJhbXN0ZWQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU3RldmVuIiwiZmFtaWx5IjoiU2Fkcm8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiR2VvZmZyZXkiLCJmYW1pbHkiOiJTY2hsYWRvdyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBbmdlbGEgRGUiLCJmYW1pbHkiOiJQYWxtYeKAkERvdyJ9fV0sImlkcyI6eyJpc3NuIjpbIjEwMTUtMTYyMSIsIjE0MjAtOTA1NSJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMTAwNy9zMDAwMjctMDI0LTAxMDQ1LTIiLCJvcGVuYWxleCI6Ilc0MzkyNzEzMzk5IiwiZXhhIjoiaHR0cHM6Ly9saW5rLnNwcmluZ2VyLmNvbS9hcnRpY2xlLzEwLjEwMDcvczAwMDI3LTAyNC0wMTA0NS0yP2Vycm9yPWNvb2tpZXNfbm90X3N1cHBvcnRlZCZjb2RlPTgzY2Y4ZDg4LWM5YTAtNGNkZC04MjU2LTFhODVlNmFiZGFjMyJ9LCJ0aXRsZSI6IkludGVybmFsIHBob3NwaG9ydXMgbG9hZGluZyBhbHRlcnMgbnV0cmllbnQgbGltaXRhdGlvbiBhbmQgY29udHJpYnV0ZXMgdG8gY3lhbm9iYWN0ZXJpYWwgYmxvb21zIGluIGEgcG9seW1pY3RpYyBsYWtlIiwiYWJzdHJhY3QiOiJBYnN0cmFjdCBDbGVhciBMYWtlLCBhIG1lZGl1bS1zaXplZCBoeXBlcmV1dHJvcGhpYywgcG9seW1pY3RpYyBsYWtlIGluIG5vcnRoZXJuIENhbGlmb3JuaWEsIGhhcyBoYWQgcmVjdXJyaW5nIGhhcm1mdWwgY3lhbm9iYWN0ZXJpYSBibG9vbXMgKEhDQnMpIGZvciBvdmVyIGEgY2VudHVyeSBkZXNwaXRlIHJlZHVjdGlvbnMgaW4gZXh0ZXJuYWwgcGhvc3Bob3J1cyAoUCkgbG9hZGluZ3MuIEludGVybmFsIFAgbG9hZGluZyBjYW4gYWx0ZXIgbnV0cmllbnQgYXZhaWxhYmlsaXR5IGFuZCBsaW1pdGF0aW9uIHN1cHBvcnRpbmcgSENCcyBidXQgaXMgcmFyZWx5IHF1YW50aWZpZWQgb3IgY29tcGFyZWQgd2l0aCBleHRlcm5hbCBsb2Fkcy4gV2UgaGF2ZSBxdWFudGlmaWVkIGV4dGVybmFsIFAgbG9hZHMgZnJvbSAyMDE5IHRvIDIwMjEgZm9yIHRoZSB0aHJlZSBtYWluIHRyaWJ1dGFyaWVzIChhY2NvdW50aW5nIGZvciA0NiUgb2YgdGhlIGZsb3cpIGFuZCBpbnRlcm5hbCBsb2FkaW5ncyB1c2luZyB0d28gbWV0aG9kczogYSBQIG1hc3MgYmFsYW5jZSBhbmQgbW9kZWxlZCByZWxlYXNlIHJhdGVzIG9mIHNvbHVibGUgcmVhY3RpdmUgcGhvc3Bob3J1cyBmcm9tIG94aWMgYW5kIGFub3hpYyBzZWRpbWVudHMuIEluIGFkZGl0aW9uLCB3ZSBjb21iaW5lZCBoaWdoLWZyZXF1ZW5jeSBpbiBzaXR1IG1lYXN1cmVtZW50cyBvZiB3YXRlciB0ZW1wZXJhdHVyZSBhbmQgZGlzc29sdmVkIG94eWdlbiwgZGlzY3JldGUgZ3JhYiBzYW1wbGluZyBmb3IgbnV0cmllbnQgY2hlbWlzdHJ5LCBhbmQgcmVtb3RlIHNlbnNpbmcgdG8gZXhwbG9yZSB0aGUgcG90ZW50aWFsIGRyaXZlcnMgb2YgdGhlIG9ic2VydmVkIHZhcmlhYmlsaXR5IGFuZCBwcm92aWRlIGEgY29tcHJlaGVuc2l2ZSB2aWV3IG9mIHRoZSBzcGF0aW90ZW1wb3JhbCBkeW5hbWljcyBvZiBIQ0JzLiBCeSB1bmRlcnN0YW5kaW5nIHRoZSByZWxhdGl2ZSBjb250cmlidXRpb24gb2YgZXh0ZXJuYWwgYW5kIGludGVybmFsIG51dHJpZW50IGxvYWRpbmdzIGFuZCB0aGUgcmVsYXRpb25zaGlwIGJldHdlZW4gZW52aXJvbm1lbnRhbCBwYXJhbWV0ZXJzIGFuZCBIQ0JzLCBpbnRlcmFubnVhbCBibG9vbSB2YXJpYWJpbGl0eSBjYW4gYmUgYmV0dGVyIHByZWRpY3RlZC4gQ29tcGFyYXRpdmUgZXN0aW1hdGVzIG9mIGV4dGVybmFsIGFuZCBpbnRlcm5hbCBwaG9zcGhvcnVzIGxvYWRpbmcgaW5kaWNhdGUgdGhhdCBpbnRlcm5hbCBzb3VyY2VzIGFjY291bnRlZCBmb3IgNzDigJM5NSUgb2YgdGhlIHRvdGFsIFAgaW5wdXQgaW50byB0aGUgc3lzdGVtIGR1cmluZyB0aGUgc3R1ZHkgcGVyaW9kLiBDb250cmFyeSB0byBvdGhlciBsYWtlcywgdGhlIGludGVuc2l0eSBvZiB0aGUgc3VtbWVyIGJsb29tIHNlYXNvbiB3YXMgY29ycmVsYXRlZCB0byB0aGUgdGltaW5nIGFuZCBkdXJhdGlvbiBvZiBhbm94aWEgcmF0aGVyIHRoYW4gdGhlIG1hZ25pdHVkZSBvZiBzcHJpbmcgcnVub2ZmLiBJbnRlcm5hbGx5IHJlbGVhc2VkIFAgc2hpZnRlZCB0aGUgc3lzdGVtIGZyb20gcGhvc3Bob3J1cyB0byBuaXRyb2dlbiBsaW1pdGF0aW9uIGR1cmluZyB0aGUgc3VtbWVyLCBwb3RlbnRpYWxseSBmYXZvcmluZyB0aGUgcHJvbGlmZXJhdGlvbiBvZiBuaXRyb2dlbi1maXhpbmcgY3lhbm9iYWN0ZXJpYS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAwNy9zMDAwMjctMDI0LTAxMDQ1LTIiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vbGluay5zcHJpbmdlci5jb20vY29udGVudC9wZGYvMTAuMTAwNy9zMDAwMjctMDI0LTAxMDQ1LTIucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjQtMDMtMTIiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTExIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJBcXVhdGljIFNjaWVuY2VzIiwiaXNzdWUiOjIsInZvbHVtZSI6ODYsImltcGFjdEZhY3RvciI6IjIuODQifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiU3ByaW5nZXIgU2NpZW5jZStCdXNpbmVzcyBNZWRpYSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NzUsImNpdGVkQnlDb3VudCI6MjF9fSwicXVvdGUiOiJDb250cmFyeSB0byBvdGhlciBsYWtlcywgdGhlIGludGVuc2l0eSBvZiB0aGUgc3VtbWVyIGJsb29tIHNlYXNvbiB3YXMgY29ycmVsYXRlZCB0byB0aGUgdGltaW5nIGFuZCBkdXJhdGlvbiBvZiBhbm94aWEgcmF0aGVyIHRoYW4gdGhlIG1hZ25pdHVkZSBvZiBzcHJpbmcgcnVub2ZmLiBJbnRlcm5hbGx5IHJlbGVhc2VkIFAgc2hpZnRlZCB0aGUgc3lzdGVtIGZyb20gcGhvc3Bob3J1cyB0byBuaXRyb2dlbiBsaW1pdGF0aW9uIGR1cmluZyB0aGUgc3VtbWVyLCBwb3RlbnRpYWxseSBmYXZvcmluZyB0aGUgcHJvbGlmZXJhdGlvbiBvZiBuaXRyb2dlbi1maXhpbmcgY3lhbm9iYWN0ZXJpYS4gLSBbQ2FsY2l1bSBhbmQgcGhvc3BoYXRlIG1ldGFib2xpYyBkaXNvcmRlcnNdKGh0dHBzOi8vbGluay5zcHJpbmdlci5jb20vc3ViamVjdHMvY2FsY2l1bS1hbmQtcGhvc3BoYXRlLW1ldGFib2xpYy1kaXNvcmRlcnMpIEV1dHJvcGhpY2F0aW9uIHBvc2VzIGEgZ3JhdmUgdGhyZWF0IHRvIGxlbnRpYyBlY29zeXN0ZW1zIGdsb2JhbGx5LCBwcm9tb3RpbmcgdGhlIHByb2xpZmVyYXRpb24gb2YgaGFybWZ1bCBjeWFub2JhY3RlcmlhIGJsb29tcyAoSENCcykgdGhhdCByZWR1Y2UgYXF1YXRpYyBiaW9kaXZlcnNpdHksIHlpZWxkaW5nIGluY3JlYXNlZCB3YXRlciB0cmVhdG1lbnQgY29zdHMsIGFuZCB0aHJlYXRlbmluZyBodW1hbiBoZWFsdGggdGhyb3VnaCBibG9vbS1wcm9kdWNlZCBjeWFub3RveGlucyAoQXLDoW96IGV0IGFsLiJ9XQ%3D%3D. However, this ephemeral bounty masks a critical vulnerability: nighttime respiration by prolific algal biomass and heterotrophic bacteria rapidly depletes DO, often plummeting concentrations below 5 mg/L before dawn(Haxthausen, 2021)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkthcmwgUmFzbXVzIEF1Z3VzdCIsImZhbWlseSI6IkhheHRoYXVzZW4ifX1dLCJpZHMiOnsiaXNzbiI6W10sImlzYm4iOltdLCJtYWciOiIzMTc3MTEzMjA3Iiwib3BlbmFsZXgiOiJXMzE3NzExMzIwNyJ9LCJ0aXRsZSI6IlNlZGltZW50IG1pY3JvYmlhbCBmdWVsIGNlbGxzIGZvciBsYWtlIHJlc3RvcmF0aW9uIiwiYWJzdHJhY3QiOiJFdXRyb3BoaWNhdGlvbiBvZiBsYWtlcyBpcyB1YmlxdWl0b3VzLCBhcyAzMC00MCAlIG9mIHRoZSB3b3JsZOKAmXMgbGFrZXMgYXJlIGV1dHJvcGhpYywgYW5kIGNvbmRpdGlvbnMgY29udGludW91c2x5IGRldGVyaW9yYXRpbmcuIEV1dHJvcGhpY2F0aW9uIGlzIGEgY29uZGl0aW9uIG9mIG92ZXJmZXJ0aWxpemF0aW9uLCB3aGVyZSBhbGwgcmVxdWlyZW1lbnRzIGZvciByYXBpZCBncm93dGggb2YgYWxnYWUgYXJlIG1ldCwgcmVzdWx0aW5nIGluIGFsZ2FsIGJsb29tcy4gVGhpcyBjYXVzZXMgYSBzZWxmLXN1c3RhaW5lZCwgdmljaW91cyBjeWNsZSB3aGVyZSBwb29yIGVjb2xvZ2ljYWwgY29uZGl0aW9ucyBhcmUgbWFpbnRhaW5lZCB1bnRpbCBudXRyaWVudHMgYXJlIGxvc3QgZnJvbSB0aGUgbGFrZSBvdmVyIG1hbnkgeWVhcnMuIExha2UgcmVzdG9yYXRpb24gc2Vla3MgdG8gYnJlYWsgdGhpcyB2aWNpb3VzIGN5Y2xlLiBJbmFkZXF1YXRlIGF2YWlsYWJpbGl0eSBvZiBhbnkgc2luZ2xlIG51dHJpZW50IGluIHdpbGwgbGltaXQgYWxnYWwgZ3Jvd3RoLCB0eXBpY2FsbHkgcGhvc3Bob3J1cyBpcyB0YXJnZXRlZC4gVGhlIHNjb3BlIG9mIHRoaXMgUGhEIHRoZXNpcyB3YXMgdG8gZGV2ZWxvcCwgdGVzdCBhbmQgb3B0aW1pemUgc2VkaW1lbnQgbWljcm9iaWFsIGZ1ZWwgY2VsbHMgKFNNRkMpIGFzIGEgbm92ZWwgbWV0aG9kIGZvciBsYWtlIHJlc3RvcmF0aW9uLiBUaGUgdGhlc2lzIHNwYW5zIHRoZSB0d28gbWFpbiB0b3BpY3Mgb2YgU01GQ3MgYW5kIGxha2UgcmVzdG9yYXRpb24gdGVjaG5pcXVlcy4gTGFrZSByZXN0b3JhdGlvbiB0ZWNobmlxdWVzIGFyZSByZXZpZXdlZCwgcHJvdmlkaW5nIHVuZGVyc3RhbmRpbmcgb2YgdGhlIHVuZGVybHlpbmcgbWVjaGFuaXNtcy4gU01GQ3MgYXJlIGFwcHJvYWNoZWQgZnJvbSBhbiBlbmdpbmVlcmluZyBwZXJzcGVjdGl2ZSBhbmQgYSBmb2N1cyBvbiBsYWtlIHJlbWVkaWF0aW9uLiBUaGUgc3BlY2lmaWMgb2JqZWN0aXZlcyB3ZXJlIHRvIHRlc3QgYSBsb3ctY29zdCBlbGVjdHJvZGUgbWF0ZXJpYWwgZm9yIGl0cyBkdXJhYmlsaXR5LCBwcmFjdGljYWxpdHksIGFuZCBwZXJmb3JtYW5jZS4gUGVyZm9ybWFuY2Ugd2FzIGFzc2Vzc2VkIGFzIHRoZSBhYmlsaXR5IHRvIGxpbWl0IHBob3NwaG9ydXMgcmVsZWFzZSBmcm9tIHNlZGltZW50cywgaW5oaWJpdCBtZXRoYW5lIHJlbGVhc2UsIGFuZCB0aGUgYWRkZWQgYmVuZWZpdHMgb2YgZGVncmFkaW5nIHBvbGx1dGFudHMsIGFuZCBnZW5lcmF0aW5nIGVsZWN0cmljaXR5LiBQcm9vZiBvZiBjb25jZXB0IHdhcyBzaG93biBpbiBsYWJvcmF0b3J5IHJlYWN0b3JzICgxIEwpLCB0aGUgZWZmZWN0IG9mIHVwc2NhbGluZyB0byAyMDAgTCBpbiBuYXR1cmFsIGVudmlyb25tZW50IHdhcyBzdHVkaWVkIGluIG91dGRvb3IgbWVzb2Nvc21zLCBhbmQgdGhlbiB0ZXN0ZWQgYXQgcGlsb3Qgc2NhbGUgaW4gYSBldXRyb3BoaWMgZnJlc2h3YXRlciBsYWtlLiBQaWxvdCBpbnN0YWxsYXRpb25zIHdlcmUgdGhlIGxhcmdlc3QgaW4gdGhlIHdvcmxkIG9mIHRoZWlyIGtpbmQgYW5kIHdlcmUgb3BlcmF0ZWQgZm9yIDIgeWVhcnMuIFN0YWlubGVzcyBzdGVlbCB3YXMgc2VsZWN0ZWQgZm9yIHRlc3RpbmcgYXMgZWxlY3Ryb2RlIG1hdGVyaWFsIGR1ZSB0byBpdHMgbG93IGNvc3QsIGR1cmFiaWxpdHksIGFuZCBhdmFpbGFiaWxpdHkuIFBhaXJzIG9mIGVsZWN0cm9kZXMgd2VyZSBlbWJlZGRlZCBpbiB0aGUgc2VkaW1lbnQgKHRoaXMgaXMgY2FsbGVkIHRoZSBhbm9kZSkgYW5kIHN1c3BlbmRlZCBpbiB0aGUgb3Zlcmx5aW5nIHdhdGVyICh0aGlzIGlzIGNhbGxlZCB0aGUgY2F0aG9kZSkuIFRoZSB0d28gZWxlY3Ryb2RlcyB3ZXJlIGNvbm5lY3RlZCBhY3Jvc3MgYW4gZXh0ZXJuYWwgcmVzaXN0b3Igd2l0aCBlbGVjdHJpY2FsIHdpcmVzLiBNaWNyb2JlcyBpbiB0aGUgc2VkaW1lbnQgdXNlIHRoZSBhbm9kZSBwYWlyIGFzIGVsZWN0cm9uIGFjY2VwdG9yLCBlbmFibGluZyByZXNwaXJhdGlvbiBpbiB0aGUgYW5veGljIHNlZGltZW50LiBFbGVjdHJvbnMgdHJhdmVsIGZyb20gdGhlIHNlZGltZW50IHZpYSB0aGUgZWxlY3RyaWNhbCBjb25uZWN0b3IgdG8gdGhlIG92ZXJseWluZyBjYXRob2RlIHdoZXJlIG94eWdlbiBpbiB0aGUgd2F0ZXIgaXMgdGhlIHRlcm1pbmFsIGVsZWN0cm9uIGFjY2VwdG9yIGNvbXBsZXRpbmcgdGhlIGNpcmN1aXQuIEFzIGFuIGFkZGVkIGJlbmVmaXQsIGRlZ3JhZGF0aW9uIG9mIHBvbGx1dGFudHMgdXNpbmcgU01GQyB3YXMgdGVzdGVkIGluIG1hcmluZSBoYXJib3VyIHNlZGltZW50cywgYnV0IG5vIGNvbmNsdXNpdmUgcmVzdWx0cyBvYnRhaW5lZC4gTGFib3JhdG9yeSByZWFjdG9ycyB3ZXJlIG9wZXJhdGVkIGZvciBtb3JlIHRoYW4gMjAwIGRheXMgdXNpbmcgcGV0cm9sZXVtIGFzIHNvbGUgY2FyYm9uIHNvdXJjZSwgY29uZmlybWluZyBvaWwgZGVncmFkYXRpb24gYnkgU01GQ3MuIEEgcmVtb3ZhbCByYXRlIG9mIDAuOSBnIHBldHJvbGV1bSBtLTMgZC0xIHJlYWN0b3Igdm9sdW1lIHdhcyBlc3RpbWF0ZWQgZnJvbSBtZWFzdXJlZCBlbGVjdHJpY2FsIG91dHB1dC4gVGhpcyB3YXMgZG9uZSBieSBhc3N1bXB0aW9uIG9mIHRoZSBjaGVtaWNhbCByZWFjdGlvbiBvY2N1cnJpbmcsIGZyb20gdGhpcyB0aGUgbnVtYmVyIG9mIGVsZWN0cm9ucyB0cmFuc2ZlcnJlZCBmcm9tIG9pbCB0byB3YXRlciB3YXMga25vd24gYW5kIGNvdWxkIGJlIGNvcnJlbGF0ZWQgdG8gdGhlIG1lYXN1cmVkIGZsb3cgb2YgZWxlY3Ryb25zIChjdXJyZW50KSB0aHJvdWdoIHRoZSBleHRlcm5hbCBjaXJjdWl0LiBTdGFpbmxlc3Mgc3RlZWwgZWxlY3Ryb2RlcyB3ZXJlIG9wZXJhdGVkIGluIGZyZXNod2F0ZXIgbGFrZSBlbnZpcm9ubWVudHMgZm9yIG1vcmUgdGhhbiA0IHllYXJzIHdpdGggbWluaW1hbCBkYW1hZ2UgdG8gdGhlIG1hdGVyaWFsLiBUaGUgZWxlY3Ryb2RlIGlzIHRoZSBtYWluIG1hdGVyaWFsIGNvc3QgaW4gc3VjaCBzeXN0ZW1zLiBIb3dldmVyLCBBSVNJMzE2IHN0YWlubGVzcyBzdGVlbCBzaW1pbGFyIHRvIHdoYXQgd2FzIHVzZWQgaW4gdGhpcyB0aGVzaXMgd2FzIGF0IHRoZSB0aW1lIG9mIHdyaXRpbmcgYXZhaWxhYmxlIGF0IGJ1bGsgY29zdCBvZiAwLjA5MSDigqwgbS0yLCBjb3JyZXNwb25kaW5nIHRvIDkxMCDigqwgYWNyZS0xLCBzaWduaWZpY2FudGx5IGxvd2VyIHRoYW4gb3RoZXIgY29tbW9uIG1hdGVyaWFscy4gQ2F0aG9kZXMgYXQgbGFyZ2Ugc2NhbGUgY291bGQgYmUgbW92ZWQgaG9yaXpvbnRhbGx5IGFuZCB2ZXJ0aWNhbGx5IHdpdGggbWlub3IgaW1wYWN0IHRvIHBlcmZvcm1hbmNlLiBQaWxvdCBzY2FsZSBTTUZDcyBnZW5lcmF0ZWQgMC4zODMgbVcgbS0yIHN1c3RhaW5lZCBhbmQgMjcuNCBtVyBtLTIgcGVhayBvdXRwdXQuIFRoZSBzeXN0ZW0gd2FzIGxpbWl0ZWQgYnkgdGhlIGNhdGhvZGUgc3VyZmFjZSBhcmVhLCBtZWFuaW5nIG11bHRpLWxheWVyIGNhdGhvZGVzIHNob3VsZCBiZSB0ZXN0ZWQgaW4gdGhlIGZ1dHVyZSBpZiBlbGVjdHJpY2FsIHBvd2VyIHByb2R1Y3Rpb24gaXMgdGhlIGdvYWwuIFRoZSBjYXRob2RlIHNob3VsZCBiZSBwbGFjZWQgYXQgYSBkZXB0aCB0aGF0IGVuc3VyZXMgaGlnaCBkaXNzb2x2ZWQgb3h5Z2VuIGNvbmNlbnRyYXRpb25zIHRvIGF2b2lkIHRoZSBzeXN0ZW0gdG8gYmUgbGltaXRlZC4gSG93ZXZlciwgdG9vIGhpZ2ggZWxlY3RyaWNpdHkgZ2VuZXJhdGlvbiB3YXMgZm91bmQgY291bnRlcnByb2R1Y3RpdmUgZm9yIHJlbWVkaWF0aW9uIHB1cnBvc2VzIGJlY2F1c2UgdGhpcyB3b3VsZCBpbmNyZWFzZSBzZWRpbWVudCB0dXJub3ZlciByYXRlLCByZWxlYXNpbmcgcG9sbHV0YW50cyBzdG9yZWQgaW4gdGhlIHNlZGltZW50cy4gUCBiaW5kaW5nIHdhcyBub3QgZm91bmQgdG8gcmVxdWlyZSBoaWdoIHR1cm5vdmVyIG9mIHNlZGltZW50IG9yZ2FuaWMgbWF0dGVyLiBUaGUgdHVybm92ZXIgY2FuIGJlIGNvbnRyb2xsZWQgYnkgY2hhbmdpbmcgdGhlIGV4dGVybmFsIHJlc2lzdGFuY2UuIEhpZ2ggcmVzaXN0YW5jZSBsaW1pdHMgdGhlIGVsZWN0cmljYWwgcGVyZm9ybWFuY2UsIGxvdyByZXNpc3RhbmNlIGluY3JlYXNlcyBzZWRpbWVudCB0dXJub3ZlciBhbmQgcmlza3MgZGVzdGFiaWxpemluZyB0aGUgc3lzdGVtLiBUbyBhY2hpZXZlIHJlZHVjZWQgY29uY2VudHJhdGlvbnMgb2YgcGhvc3BoYXRlIGluIHRoZSB3YXRlciBjb2x1bW4sIGFub2RlcyBtdXN0IGJlIHBsYWNlZCBpbiB0aGUgc2VkaW1lbnQgc3VyZmFjZSBzaW5jZSBhIGZldyBtaWxsaW1ldHJlcyBvZiBzZWRpbWVudCBtYXkgY29udGFpbiBhbmQgcmVsZWFzZSBlbm91Z2ggUCB0byBzdXN0YWluIGV1dHJvcGhpY2F0aW9uLiBQaG9zcGhhdGUgd2FzIG1lYXN1cmVkIGluIHRoZSB3YXRlciBjb2x1bW4gbmVhciB0aGUgc2VkaW1lbnQsIGluIHN5c3RlbXMgd2l0aCBhbmQgd2l0aG91dCBlbGVjdHJvZGVzLiBQaG9zcGhhdGUgY29uY2VudHJhdGlvbnMgd2VyZSBzdWNjZXNzZnVsbHkgcmVkdWNlZCBieSB1cCB0byA5NSAlIGluIDItY2hhbWJlciBsYWJvcmF0b3J5IHJlYWN0b3JzLCBhbmQgYnkgdXAgdG8gOTggJSBpbiB0aGUgcGlsb3QgaW5zdGFsbGF0aW9uLiBJbiB0aGUgcGlsb3QgZXhwZXJpbWVudCwgcGhvc3BoYXRlIGNvbmNlbnRyYXRpb25zIGFib3ZlIGVsZWN0cm9kZXMgd2VyZSBsb3dlciBpbiBhbGwgbWVhc3VyZW1lbnRzLCBjb21wYXJlZCB0byBjb250cm9scy4gU2VkaW1lbnQgcmVkb3ggcG90ZW50aWFscyBpbmNyZWFzZWQgZnJvbSAtNDAwIG1WIHZzIHN0YW5kYXJkIGh5ZHJvZ2VuIGVsZWN0cm9kZSAoU0hFKSB0byAtMTc1IG1WIHZzIFNIRSwgY29ycmVzcG9uZGluZyB0byBhIG1vdmUgZnJvbSBhIG1ldGhhbm9nZW5ldGljIGRvbWFpbiB0byBzdWxwaHVyIHJlZHVjaW5nLiBBbmFseXNpcyBvZiB0aGUgbWljcm9iaWFsIGNvbW11bml0eSBpbmRpY2F0ZWQgZW5oYW5jZWQgc3VscGh1ciBjeWNsaW5nIHdoZXJlIGVsZWN0cm9kZXMgb3hpZGl6ZSBzdWxwaGlkZSB3aGljaCB0aGUgYmFjdGVyaWEgYWdhaW4gcmVkdWNlIHRvIG9idGFpbiBlbmVyZ3kuIEl0IHdhcyBoeXBvdGhlc2lzZWQgdGhhdCBTTUZDIHByb21vdGUgaGlnaGVyIHJlZG94IHBvdGVudGlhbHMgYnkgZmFjaWxpdGF0aW5nIG94aWRhdGlvbiBvZiBzdWxwaHVyIGFuZCBpcm9uIHNwZWNpZXMsIGluaGliaXRpbmcgSDJTIGZvcm1hdGlvbiBhbmQgRmUgc2NhdmVuZ2luZyBieSBGZVMgZm9ybWF0aW9uLCB0aHVzIGFsbG93aW5nIGZvciBQIGJpbmRpbmcuIEEgbG93ZXIgcmVsYXRpdmUgYWJ1bmRhbmNlIG9mIG1ldGhhbm9nZW5zIG9uIGVsZWN0cm9kZXMgY29tcGFyZWQgdG8gY29udHJvbHMgc3VnZ2VzdHMgc3VwcHJlc3Npb24gb3Igb3V0Y29tcGV0aW5nIG9mIG1ldGhhbm9nZW5zLlF1YW50aWZpY2F0aW9uIG9mIG1ldGhhbmUgcHJvZHVjdGlvbiB3YXMgZG9uZSBvbiB0aGUgYmFzaXMgb2YgYSBDSDQ6Q08yIHJhdGlvIHRvIGFsbG93IGNvbXBhcmFiaWxpdHkgYmV0d2VlbiBzeXN0ZW1zLiBNZXRoYW5lIGVtaXNzaW9uIHdhcyByZWR1Y2VkIGJ5IHVwIHRvIDk5ICUgYW5kIG1haW50YWluZWQgaW5oaWJpdGlvbiBmb3Igc2V2ZXJhbCBtb250aHMgaW4gbGFib3JhdG9yeSBzeXN0ZW1zIHdpdGggZWxlY3Ryb2Rlcy4gSW4gdGhlIHBpbG90IGluc3RhbGxhdGlvbnMgbm8gZ2FzIHNhbXBsZXMgY291bGQgYmUgY29sbGVjdGVkLCBhbmQgbm8gQ08yOkNINCByYXRpb3Mgd2VyZSBvYnRhaW5lZC4gQWVyYXRpb24gaXMgdGhlIGN1cnJlbnQgcmVzdG9yYXRpb24gdGVjaG5pcXVlIHRoYXQgbW9zdCBjbG9zZWx5IHJlc2VtYmxlcyB0aGUgcHJvcG9zZWQgU01GQyB0ZWNobmlxdWUuIEFlcmF0aW9uIGhhcyBwcmV2aW91c2x5IGZhaWxlZCB0byByZWR1Y2UgUCByZWxlYXNlIGluIHNldmVyYWwgc2VwYXJhdGUgaW52ZXN0aWdhdGlvbnMuIFNNRkNzIGFzIGEgcmVtZWRpYXRpb24gdG9vbCB3YXMgZm91bmQgdG8gb2ZmZXIgc2V2ZXJhbCBiZW5lZml0cyBvdmVyIGNvbnZlbnRpb25hbCBtZXRob2RzOiBTTUZDcyBvZmZlcnMgY29udGludW91cyBlZmZlY3QsIHdoaWxlIG90aGVyIHRlY2huaXF1ZXMgcmVxdWlyZSBjb250aW51b3VzIG9yIHJlcGVhdGVkIHRyZWF0bWVudC4gU01GQ3MgY2FuIGJlIGNvbnRyb2xsZWQgaW4gcmVhbC10aW1lIGJ5IGFsdGVyaW5nIHRoZSBleHRlcm5hbCByZXNpc3RvciBhbmQgY2FuIGJlIGVhc2lseSBtb25pdG9yZWQgYnkgbWVhc3VyaW5nIHRoZSBwcm9kdWNlZCBlbGVjdHJpY2FsIG91dHB1dC4gSWYgaW1wbGVtZW50ZWQgYXQgbGFyZ2Ugc2NhbGUgaXQgaXMgZXhwZWN0ZWQgdGhhdCB0aGUgZWxlY3Ryb2RlIGZpZWxkcyB3aWxsIGhlbHAgbWFpbnRhaW4gbG93IHBob3NwaGF0ZSBjb25jZW50cmF0aW9ucyBpbiB0aGUgd2F0ZXIgZm9yIGRlY2FkZXMgd2l0aCBtaW5pbWFsIGludGVydmVudGlvbiwgYW5kIGFmdGVyIHNvbWUgdGltZSwgYmVudGhpYyBwbGFudHMgYXJlIGV4cGVjdGVkIHRvIHJldHVybiwgdGh1cyBtYWludGFpbmluZyB0aGUgYmFsYW5jZSBoZW5jZSBmb3J0aC4iLCJ1cmwiOiJodHRwczovL2xvY2FsLmZvcnNrbmluZ3Nwb3J0YWwuZGsvbG9jYWwvZGtpLWNnaS93cy9jcmlzLWxpbms%2Fc3JjPWR0dSZpZD1kdHUtZDY2ZDMzNzAtMjU2Zi00ODg1LWEwOWUtMzM2YTkyNGY0YmVkJnRpPVNlZGltZW50JTIwbWljcm9iaWFsJTIwZnVlbCUyMGNlbGxzJTIwZm9yJTIwbGFrZSUyMHJlc3RvcmF0aW9uIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2JhY2tlbmQub3JiaXQuZHR1LmRrL3dzL2ZpbGVzLzI0ODA2OTU4OS9UaGVzaXNfb25saW5lX3ZlcnNpb25fS2FybF9IYXh0aGF1c2VuLnBkZiIsImlzc3VlZERhdGUiOiIyMDIxLTAxLTAxIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0wOCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiUmVzZWFyY2ggUG9ydGFsIERlbm1hcmsiLCJpbXBhY3RGYWN0b3IiOiIwLjA0In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6NzZ9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiVGVjaG5pY2FsIFVuaXZlcnNpdHkgb2YgRGVubWFyayJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MCwiY2l0ZWRCeUNvdW50IjoxfX0sInF1b3RlIjoiRXh0ZXJuYWwgaW5wdXRzIGluY3JlYXNlIFAgY29uY2VudHJhdGlvbiBpbiAgdGhlIHdhdGVyLCBjYXVzaW5nIGFsZ2FsIGJsb29tcyB0aGF0IHNoYWRvdyB0aGUgbGFrZSBib3R0b20sIGFuZCBiZW50aGljIG1hYy1yb3BoeXRlcyByZWNlZGUuIFByZWRhdG9yeSBmaXNoIGFyZSB1bmFibGUgdG8gaHVudCwgYW5kIHRoZWlyIGZvb2Qgc291cmNlIGlzICBkaXNydXB0ZWQgc3VjaCBhcyBmaXNoIGZlZWRpbmcgb24gYmVudGhpYyBpbnZlcnRlYnJhdGVzIGFuZCBib3R0b20tZHdlbGxpbmcgYW5pbWFscyBhcmUgc3VmZm9jYXRlZCAoVmFuZGVyIFphbmRlbiBhbmQgVmFkZWJvbmNvZXVyIDIwMDIpLCBzbyB0aGF0IHBsYW5rdGl2b3JvdXMgZmlzaCBzcGVjaWVzIGRvbWluYXRlLiBUaGlzIGluY3JlYXNlcyBwcmVkYXRpb24gb24gem9vIHBsYW5rdG9uICBhbmQgcmVkdWNlcyBncmF6aW5nIG9uIGFsZ2FlLiBBbGdhZSBwcm9saWZlcmF0ZSwgZGllLCBhbmQgc2luayB0byB0aGUgYm90dG9tLCAgY29uc3VtaW5nIG94eWdlbiB3aGVuIGRlZ3JhZGVkLCB3aGljaCBsZWFkcyB0byBveHlnZW4gZGVmaWNpdCBpbiB0aGUgYm90LXRvbSwga2lsbGluZyBmaXNoIChXdXJ0c2JhdWdoIGV0IGFsLiAyMDE5KS4gRmlndXJlIDI6IFZpY2lvdXMgY3ljbGVzIG9mIGV1dHJvcGhpY2F0aW9uLiBMZWZ0OiBJbnRlcmxpbmtlZCBwcm9jZXNzZXMgc3VzdGFpbmluZyBldXRyb3BoaS1jYXRpb24gaW4gYSBsYWtlLiBCbHVlOiBwcm9jZXNzZXMgaW4gd2F0ZXIsIGdyZWVuOiBwcm9jZXNzZXMgaW52b2x2aW5nIG1pY3JvYWxnYWUsIGdyZXk6IHNlZC1pbWVudCByZWxhdGVkIHByb2Nlc3NlcywgeWVsbG93OiBwcm9jZXNzZXMgaW52b2x2aW5nIGZhdW5hLiBSaWdodCwgZGFzaGVkIGxpbmU6IFRoZSBvcmRlciBpbiB3aGljaCBlbGVjdHJvbiBhY2NlcHRvcnMgYXJlIGNvbnN1bWVkIGluIHRoZSBhYnNlbmNlIG9mIE8yIHNob3duIGFzIGNvbmNlbnRyYXRpb24gdmVyc3VzICBkZXB0aCBpbiBzZWRpbWVudCwgYW5kIGF0IHdoaWNoIHBvdGVudGlhbCB0aGlzIG9jY3VycyAoQmVybmVyIDE5ODEsIGJ5IGNvdXJ0ZXN5IG9mIHRoZSBzb2NpZXR5ICBmb3Igc2VkaW1lbnRhcnkgZ2VvbG9neSkuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxNywiYmJveCI6eyJ0b3AiOjc5LjkzMzE1MTI0NTExNzE5LCJsZWZ0IjoxNC4yOTU1MDM2MTYzMzMwMDgsIndpZHRoIjo3MS45Mzg5MzQzMjYxNzE4OCwiaGVpZ2h0IjoyLjA1MzQ3NzI4NzI5MjQ4MDV9fSx7InBhZ2VOdW1iZXIiOjE3LCJiYm94Ijp7InRvcCI6ODEuOTcxMzQzOTk0MTQwNjIsImxlZnQiOjE0LjI5NTUwMzYxNjMzMzAwOCwid2lkdGgiOjcxLjMzMzAwMDE4MzEwNTQ3LCJoZWlnaHQiOjIuMDUzNDc3Mjg3MjkyNDgwNX19LHsicGFnZU51bWJlciI6MTcsImJib3giOnsidG9wIjo4NC4wMjM3OTYwODE1NDI5NywibGVmdCI6MTQuMjk1NTAzNjE2MzMzMDA4LCJ3aWR0aCI6NzEuODEyMDE5MzQ4MTQ0NTMsImhlaWdodCI6Mi4wNTM0NzcyODcyOTI0ODA1fX0seyJwYWdlTnVtYmVyIjoxNywiYmJveCI6eyJ0b3AiOjg2LjA2MTk5NjQ1OTk2MDk0LCJsZWZ0IjoxNC4yOTU1MDM2MTYzMzMwMDgsIndpZHRoIjo3MS45NTczNTkzMTM5NjQ4NCwiaGVpZ2h0IjoyLjA1MzQ3NzI4NzI5MjQ4MDV9fSx7InBhZ2VOdW1iZXIiOjE4LCJiYm94Ijp7InRvcCI6OS45MjY0ODc5MjI2Njg0NTcsImxlZnQiOjE0LjI5NTUwMzYxNjMzMzAwOCwid2lkdGgiOjcxLjk3NDA2MDA1ODU5Mzc1LCJoZWlnaHQiOjEwLjIzNDc3NzQ1MDU2MTUyM319LHsicGFnZU51bWJlciI6MTgsImJib3giOnsidG9wIjo0NC43MzQwNDMxMjEzMzc4OSwibGVmdCI6MTQuMjk1NTAzNjE2MzMzMDA4LCJ3aWR0aCI6NzEuODkzNDMyNjE3MTg3NSwiaGVpZ2h0IjoxMC40MDMyODk3OTQ5MjE4NzV9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI5MyJ9fV0%3D. In shallow, polymictic Indian lakes like Bellandur and Hussainsagar, exacerbated by warming temperatures and stratification, this diurnal oscillation creates hypoxic hotspots, stressing aerobic organisms and setting the stage for broader deoxygenation (Gobler et al., 2024; Kumar & Mahajan, 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkNocmlzdG9waGVyIEouIiwiZmFtaWx5IjoiR29ibGVyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlJ1dGggVy4iLCJmYW1pbHkiOiJEcmlua3dhdGVyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkEuIFcuIiwiZmFtaWx5IjoiQW50aG9ueSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJKZW5uaWZlciBBLiIsImZhbWlseSI6IkdvbGVza2kifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW5uIE1hcmllIEUuIiwiZmFtaWx5IjoiRmFtdWxhcm8tUGVjb3JhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1pY2hhZWwgQi4iLCJmYW1pbHkiOiJXYWxsYWNlIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5vcmEgUi4gVy4iLCJmYW1pbHkiOiJTdHJhcXVhZGluZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJSb25vam95IiwiZmFtaWx5IjoiSGVtIn19XSwiaWRzIjp7Imlzc24iOlsiMTY2NC0zMDJYIl0sImlzYm4iOltdLCJkb2kiOiIxMC4zMzg5L2ZtaWNiLjIwMjQuMTQ2NDY4NiIsInBtaWQiOiIzOTU2ODk5NyIsIm9wZW5hbGV4IjoiVzQ0MDQxMjA5NTIiLCJleGEiOiJodHRwczovL3d3dy5mcm9udGllcnNpbi5vcmcvam91cm5hbHMvbWljcm9iaW9sb2d5L2FydGljbGVzLzEwLjMzODkvZm1pY2IuMjAyNC4xNDY0Njg2L2Z1bGwifSwidGl0bGUiOiJTZXdhZ2UtYW5kIGZlcnRpbGl6ZXItZGVyaXZlZCBudXRyaWVudHMgYWx0ZXIgdGhlIGludGVuc2l0eSwgZGl2ZXJzaXR5LCBhbmQgdG94aWNpdHkgb2YgaGFybWZ1bCBjeWFub2JhY3RlcmlhbCBibG9vbXMgaW4gZXV0cm9waGljIGxha2VzIiwiYWJzdHJhY3QiOiJDeWFub2JhY3RlcmlhbCBoYXJtZnVsIGFsZ2FsIGJsb29tcyAoQ0hBQnMpIGFyZSBwcm9tb3RlZCBieSBleGNlc3NpdmUgbnV0cmllbnQgbG9hZGluZyBhbmQsIHdoaWxlIGZlcnRpbGl6ZXJzIGFuZCBzZXdhZ2UgYXJlIHRoZSBtb3N0IHByZXZhbGVudCBleHRlcm5hbCBudXRyaWVudCBzb3VyY2VzIGluIG1vc3Qgd2F0ZXJzaGVkcywgdGhlIGRpZmZlcmVudGlhbCBlZmZlY3RzIG9mIHRoZXNlIG51dHJpZW50IHNvdXJjZXMgb24gQ0hBQnMgYXJlIHVua25vd24uIEhlcmUsIHdlIHRyYWNrZWQgQ0hBQnMgYW5kIHBlcmZvcm1lZCBleHBlcmltZW50cyBpbiBmaXZlIGRpc3RpbmN0IGxha2VzIGFjcm9zcyB0aGUgTm9ydGhlcm4gVVMgaW5jbHVkaW5nIExha2UgRXJpZS4gRmVydGlsaXplcnMgd2l0aCBhbW1vbml1bSBhbmQgb3J0aG9waG9zcGhhdGUsIG1lbWJyYW5lICgwLjIgzrxtKS1maWx0ZXJlZCBzZXdhZ2UgKGRvbWluYXRlZCBieSByZWR1Y2VkIGZvcm1zIG9mIG5pdHJvZ2VuKSBzYW5kLWFuZCBtZW1icmFuZS1maWx0ZXJlZCBzZXdhZ2UgKGRvbWluYXRlZCBieSBuaXRyYXRlKSwgYW5kIGFuIGlub3JnYW5pYyBudXRyaWVudCBzb2x1dGlvbiBvZiBhbW1vbml1bSBhbmQgb3J0aG9waG9zcGhhdGUgd2VyZSB1c2VkIGFzIGV4cGVyaW1lbnRhbCBudXRyaWVudCBzb3VyY2VzIGZvciBDSEFCcyBhdCBOLWVxdWl2YWxlbnQsIGVudmlyb25tZW50YWxseSByZWFsaXN0aWMgY29uY2VudHJhdGlvbnMuIFBoeXRvcGxhbmt0b24gY29tbXVuaXRpZXMgd2VyZSBldmFsdWF0ZWQgZmx1b3JvbWV0cmljYWxseSwgbWljcm9zY29waWNhbGx5LCBhbmQgdmlhIGhpZ2ggdGhyb3VnaHB1dCBzZXF1ZW5jaW5nIG9mIHRoZSAxNlMgclJOQSBnZW5lLCBhbmQgbGV2ZWxzIG9mIG1pY3JvY3lzdGluIGFuZCB0aGUgzrQgMTUgTiBjb250ZW50IG9mIHBhcnRpY3VsYXRlIG9yZ2FuaWMgbml0cm9nZW4gKM60UE8gMTUgTikgd2VyZSBxdWFudGlmaWVkLiBGZXJ0aWxpemVyIGFuZCBib3RoIHNvdXJjZXMgb2Ygd2FzdGV3YXRlciBpbmNyZWFzZWQgdGhlIGFidW5kYW5jZSBvZiBjeWFub2JhY3RlcmlhIGluIGFsbCBleHBlcmltZW50cyBhY3Jvc3MgYWxsIGZpdmUgbGFrZXMgKCBwICZhbXA7bHQ7IDAuMDUgZm9yIGFsbCkgd2hlcmVhcyBlZmZlY3RzIG9uIGV1a2FyeW90aWMgcGh5dG9wbGFua3RvbiB3ZXJlIGxpbWl0ZWQuIFNhbmQtZmlsdGVyZWQgc2V3YWdlIGNvbnRhaW5lZCBsZXNzIFAsIG9yZ2FuaWMgbWF0dGVyLCBhbmQgYW1tb25pdW0gYnV0IG1vcmUgbml0cmF0ZSBhbmQgaGFkIGEgMjUlIGxlc3MgcG90ZW50IHN0aW11bGF0b3J5IGVmZmVjdCBvbiBjeWFub2JhY3RlcmlhIHRoYW4gbWVtYnJhbmUtZmlsdGVyZWQgc2V3YWdlLCBzdWdnZXN0aW5nIG5pdHJpZmljYXRpb24gbWF5IHBsYXkgYSByb2xlIGluIHJlZHVjaW5nIENIQUJzLiBGZXJ0aWxpemVyIGluY3JlYXNlZCBtaWNyb2N5c3RpbiBsZXZlbHMgYW5kIGRlY3JlYXNlZCB0aGUgzrRQTyAxNSBOIHdoZXJlYXMgd2FzdGV3YXRlciBpbmNyZWFzZWQgzrRQTyAxNSBOICggcCAmYW1wO2x0OyAwLjA1IGZvciBhbGwpLiBNaWNyb2N5c3RpcyB3YXMgdGhlIGdlbnVzIG1vc3QgY29uc2lzdGVudGx5IHByb21vdGVkIGJ5IG51dHJpZW50IHNvdXJjZXMgKCBwICZhbXA7bHQ7IDAuMDUgaW4gYWxsIGV4cGVyaW1lbnRzKSwgZm9sbG93ZWQgYnkgQ3lhbm9iaXVtICggcCAmYW1wO2x0OyAwLjA1IGluIDUwJSBvZiBleHBlcmltZW50cyksIHdpdGggaW5jcmVhc2VzIGluIE1pY3JvY3lzdGlzIGJpb21hc3MgY29uc2lzdGVudGx5IGVsaWNpdGVkIGJ5IG1lbWJyYW5lLWZpbHRlcmVkIHdhc3Rld2F0ZXIuIENvbGxlY3RpdmVseSwgcmVzdWx0cyBkZW1vbnN0cmF0ZSB0aGF0IGRpZmZlcmluZyB0eXBlcyBvZiBzZXdhZ2UgZGlzY2hhcmdlIGFuZCBmZXJ0aWxpemVycyBjYW4gcHJvbW90ZSBDSEFCIGludGVuc2l0eSBhbmQgdG94aWNpdHksIHdoaWxlIGNvbmN1cnJlbnRseSBhbHRlcmluZyBDSEFCIGRpdmVyc2l0eSBhbmQgzrRQTyAxNSBOLiBXaGlsZSBtZW1icmFuZS1maWx0ZXJlZCBzZXdhZ2UgY29uc2lzdGVudGx5IGZhdm9yZWQgTWljcm9jeXN0aXMgLCB0aGUgZGlzY2hhcmdlIG9mIHNld2FnZSB0aHJvdWdoIHNhbmRzIG11dGVkIGJsb29tIGludGVuc2l0eSBzdWdnZXN0aW5nIHNhbmQtYmVkcyBtYXkgcmVwcmVzZW50IGEgdG9vbCB0byByZW1vdmUga2V5IG51dHJpZW50cyBhbmQgcGFydGlhbGx5IG1pdGlnYXRlIENIQUJzLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzg5L2ZtaWNiLjIwMjQuMTQ2NDY4NiIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cuZnJvbnRpZXJzaW4ub3JnL2pvdXJuYWxzL21pY3JvYmlvbG9neS9hcnRpY2xlcy8xMC4zMzg5L2ZtaWNiLjIwMjQuMTQ2NDY4Ni9wZGYiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMS0wNiIsImFjY2Vzc2VkRGF0ZSI6IjIwMjYtMDIiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZyb250aWVycyBpbiBNaWNyb2Jpb2xvZ3kiLCJ2b2x1bWUiOjE1LCJpbXBhY3RGYWN0b3IiOiIzLjk5In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTQ2NDY4NiwibGFzdFBhZ2UiOjE0NjQ2ODZ9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRnJvbnRpZXJzIE1lZGlhIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo3MywiY2l0ZWRCeUNvdW50Ijo5fX0sInF1b3RlIjoiZGVyaXZlZCBudXRyaWVudHMgYWx0ZXIgdGhlIGludGVuc2l0eSwgZGl2ZXJzaXR5LCBhbmQgdG94aWNpdHkgb2YgaGFybWZ1bCBjeWFub2JhY3RlcmlhbCBibG9vbXMgLi4uIEN5YW5vYmFjdGVyaWFsIGhhcm1mdWwgYWxnYWwgYmxvb21zIChDSEFCcykgYXJlIHByb21vdGVkIGJ5IGV4Y2Vzc2l2ZSBudXRyaWVudCBsb2FkaW5nIGFuZCwgd2hpbGUgZmVydGlsaXplcnMgYW5kIHNld2FnZSBhcmUgdGhlIG1vc3QgcHJldmFsZW50IGV4dGVybmFsIG51dHJpZW50IHNvdXJjZXMgaW4gbW9zdCB3YXRlcnNoZWRzLCB0aGUgZGlmZmVyZW50aWFsIGVmZmVjdHMgb2YgdGhlc2UgbnV0cmllbnQgLi4uIG9yZ2FuaWMgbml0cm9nZW4gKM60UE8xNU4pIHdlcmUgcXVhbnRpZmllZC4gRmVydGlsaXplciBhbmQgYm90aCBzb3VyY2VzIG9mIHdhc3Rld2F0ZXIgaW5jcmVhc2VkIHRoZSBhYnVuZGFuY2Ugb2YgLi4uIHBoeXRvcGxhbmt0b24gd2VyZSBsaW1pdGVkLiBTYW5kLWZpbHRlcmVkIHNld2FnZSBjb250YWluZWQgbGVzcyBQLCBvcmdhbmljIG1hdHRlciwgYW5kIGFtbW9uaXVtIGJ1dCBtb3JlIG5pdHJhdGUgYW5kIC4uLiBkaWZmZXJpbmcgdHlwZXMgb2Ygc2V3YWdlIGRpc2NoYXJnZSBhbmQgZmVydGlsaXplcnMgY2FuIHByb21vdGUgQ0hBQiBpbnRlbnNpdHkgYW5kIHRveGljaXR5LCB3aGlsZSBjb25jdXJyZW50bHkgLi4uIHNld2FnZSB0aHJvdWdoIHNhbmRzIG11dGVkIGJsb29tIGludGVuc2l0eSBzdWdnZXN0aW5nIHNhbmQtYmVkcyBtYXkgcmVwcmVzZW50IGEgdG9vbCB0byByZW1vdmUga2V5IG51dHJpZW50cyBhbmQgLi4uICggV2F0c29uIGV0IGFsLiwgMjAxNiA7IEJ1bGxlcmphaG4gZXQgYWwuLCAyMDE2IDsgSHVpc21hbiBldCBhbC4sIDIwMTggKSBhbmQgZXhjZXNzaXZlIG51dHJpZW50IGxvYWRpbmcgaXMgdGhlIHByb2Nlc3MgbW9zdCBjb21tb25seSBjaXRlZCBhcyB0aGUgZW52aXJvbm1lbnRhbCBmYWN0b3IgcHJvbW90aW5nIENIQUJzICgifSx7Im9yaWdpbiI6NCwid29yayI6eyJ0eXBlIjo4LCJzdWJ0eXBlIjp7Im9uZW9mS2luZCI6ImFydGljbGVUeXBlIiwiYXJ0aWNsZVR5cGUiOjF9LCJjb250cmlidXRvcnMiOlt7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQYXdhbiIsImZhbWlseSI6Ikt1bWFyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFtYnJpc2ggS3VtYXIiLCJmYW1pbHkiOiJNYWhhamFuIn19XSwiaWRzIjp7Imlzc24iOlsiMjUyMy0zOTYzIiwiMjUyMy0zOTcxIl0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDA3L3M0MjQ1Mi0wMjAtMzA4Mi04IiwibWFnIjoiMzAzNTk2MDIyOCIsIm9wZW5hbGV4IjoiVzMwMzU5NjAyMjgifSwidGl0bGUiOiJUcm9waGljIHN0YXR1cyBhbmQgaXRzIHJlZ3VsYXRpbmcgZmFjdG9yIGRldGVybWluYXRpb24gYXQgdGhlIFJld2Fsc2FyIExha2UsIG5vcnRod2VzdCBIaW1hbGF5YSAoSFApLCBJbmRpYSwgYmFzZWQgb24gc2VsZWN0ZWQgcGFyYW1ldGVycyBhbmQgbXVsdGl2YXJpYXRlIHN0YXRpc3RpY2FsIGFuYWx5c2lzIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjEwMDcvczQyNDUyLTAyMC0zMDgyLTgiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vbGluay5zcHJpbmdlci5jb20vY29udGVudC9wZGYvMTAuMTAwNy9zNDI0NTItMDIwLTMwODItOC5wZGYiLCJpc3N1ZWREYXRlIjoiMjAyMC0wNi0yMiIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IlNOIEFwcGxpZWQgU2NpZW5jZXMiLCJpc3N1ZSI6Nywidm9sdW1lIjoyLCJpbXBhY3RGYWN0b3IiOiI3LjM1In0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlNwcmluZ2VyIE5hdHVyZSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NDAsImNpdGVkQnlDb3VudCI6MTl9fSwicXVvdGUiOiJDaGFuZ2luZyBxdWFsaXR5IGhhcyBjcmVhdGVkIHRoZSB3YXRlciBzY2FyY2l0eSBhbmQgIGEgaHVnZSBtYXNzIG9mIHBvcHVsYXRpb24gdGh1cyBiZWVuIGRlcHJpdmVkIGZyb20gdXNpbmcgIHRoZSBmcmVzaHdhdGVyIGZvciB2YXJpb3VzIHB1cnBvc2VzIFszLCA0XS4gSG93ZXZlciwgdGhlICBvYm5veGlvdXMgbWF0ZXJpYWxzIHRoYXQgc3RheSBpbiB3YXRlciBib2RpZXMgZm9yIGEgbG9uZy10aW1lIHBlcmlvZCBicmluZyBjaGFuZ2VzIGluIHRoZSBwaHlzaW9jaGVtaWNhbCBjaGFyLWFjdGVyaXN0aWNzIGFzIHdlbGwgYXMgdGhlIG5hdHVyYWwgc2NlbmFyaW8gb2YgdGhlIGxha2VzIFs1XS4gIFRoZSBIaW1hbGF5YW4gbGFrZXMgYXJlIHVuZGVyIHN0cmVzcyBkdWUgdG8gbWlzbWFuYWdlZCBodW1hbiBhY3Rpdml0aWVzLCB3aGljaCBpbmNyZWFzZSBudXRyaWVudHMgYW5kIHBvbGx1dGlvbiBpbiB0ZXJtcyBvZiBoZWF2eSBtZXRhbHMgdG94aWNpdHkgYW5kIGJhY3RlcmlhbCBjb250YW1pbmF0aW9uICBbNuKAkzhdLiBUaGUgbWFuYWdlbWVudCBhbmQgcHJlc2VydmF0aW9uIG9mIHRoZSBsYWtlcyBhdCAgcHJlc2VudCBoYXZlIGJlY29tZSBhIGdyZWF0IGNoYWxsZW5nZSBmb3IgZW52aXJvbm1lbnRhbC1pc3RzIGFuZCBtYW5hZ2VycyBbNSwgOV0uIFRoZSBldXRyb3BoaWNhdGlvbiBpcyBhbHNvIGEgbWFpbiBwcm9ibGVtIG9mIHRoZSAgZnJlc2h3YXRlciBsYWtlcywgYW5kIGl0IGlzIHNwZWNpZmllZCBmb3IgdGhlIGxha2VzIGhhdmluZyAgbnV0cmllbnQgZW5yaWNobWVudC4gSXQgY2F1c2VzIGV4Y2Vzc2l2ZSBncm93dGggb2YgYWxnYWUgIGFuZCBpcyBjb25zaWRlcmVkIHRvIGJlIHJlc3BvbnNpYmxlIGZvciBkZWdyYWRpbmcgdGhlICBxdWFsaXRhdGl2ZSBuYXR1cmUgb2YgdGhlIGxha2VzIFs1LCAxMF0uIFRoZSBldXRyb3BoaWNhdGlvbiAgaXMgZm9sbG93ZWQgYnkgdGhlIGFsZ2FsIGJsb29tLCB3aGljaCBnZW5lcmF0ZXMgdG94aW4sICB3b3JzZW5zIHdhdGVyIHF1YWxpdHksIGRlcGxldGVzIGRpc3NvbHZlZCBveHlnZW4gKERPKSAqIFBhd2FuIEt1bWFyLCBwYXdhbi5ldnNAZ21haWwuY29tIHwgMURlcGFydG1lbnQgb2YgRW52aXJvbm1lbnRhbCBTY2llbmNlcywgQ2VudHJhbCBVbml2ZXJzaXR5IG9mIEhpbWFjaGFsIFByYWRlc2gsIERoYXJhbXNoYWxhIDE3NjIxNSwgSW5kaWEuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NjguNTc4NDYwNjkzMzU5MzgsImxlZnQiOjguNTcxNDE4NzYyMjA3MDMxLCJ3aWR0aCI6NDAuMzI3NTE0NjQ4NDM3NSwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjcwLjE1ODk5NjU4MjAzMTI1LCJsZWZ0Ijo4LjU2OTczODM4ODA2MTUyMywid2lkdGgiOjQwLjM1NDcyNDg4NDAzMzIsImhlaWdodCI6MS41MTk4NTUyNjA4NDg5OTl9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo3MS43Mzk1NDc3Mjk0OTIxOSwibGVmdCI6OC41Njk3MzgzODgwNjE1MjMsIndpZHRoIjo0MC4zNDk2OTMyOTgzMzk4NDQsImhlaWdodCI6MS41MTk4NTUyNjA4NDg5OTl9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo3My4zMjAwOTEyNDc1NTg2LCJsZWZ0Ijo4LjU2ODA1OTkyMTI2NDY0OCwid2lkdGgiOjQwLjAyMDA4ODE5NTgwMDc4LCJoZWlnaHQiOjEuNTE5ODU1MjYwODQ4OTk5fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NzQuOTAwNjQyMzk1MDE5NTMsImxlZnQiOjguNTY4MDU5OTIxMjY0NjQ4LCJ3aWR0aCI6MzkuOTk5OTI3NTIwNzUxOTUsImhlaWdodCI6MS41MTk4NTUyNjA4NDg5OTl9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo3Ni40ODExODU5MTMwODU5NCwibGVmdCI6OC41NjgwNTcwNjAyNDE3LCJ3aWR0aCI6NDAuMzU3MDc0NzM3NTQ4ODMsImhlaWdodCI6MS41MTk4NTUyNjA4NDg5OTl9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo3OC4wNjE3Mjk0MzExNTIzNCwibGVmdCI6OC41Njk3MzY0ODA3MTI4OSwid2lkdGgiOjQwLjM1MzcxMzk4OTI1NzgxLCJoZWlnaHQiOjEuNTE5ODU1MjYwODQ4OTk5fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NjAuNjc1NzI3ODQ0MjM4MjgsImxlZnQiOjUxLjQyNzE2NTk4NTEwNzQyLCJ3aWR0aCI6NDAuMzc4NTY2NzQxOTQzMzYsImhlaWdodCI6MTguOTA1ODU1MTc4ODMzMDA4fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6ODEuOTI3MzM3NjQ2NDg0MzgsImxlZnQiOjguNDAxNDk3ODQwODgxMzQ4LCJ3aWR0aCI6NzMuOTU0Njk2NjU1MjczNDQsImhlaWdodCI6Mi42Nzg3NjI2NzQzMzE2NjV9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIxIn19XQ%3D%3D. Vertical gradients intensify the issue, with calm conditions prolonged by reduced wind speeds under climate change trapping low-DO waters near sediments, where internal phosphorus release further sustains blooms (Swann et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1pY2FoIE0uIiwiZmFtaWx5IjoiU3dhbm4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQWxpY2lhIiwiZmFtaWx5IjoiQ29ydMOpcyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBbGV4YW5kZXIgTC4iLCJmYW1pbHkiOiJGb3JyZXN0In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5pY2hvbGFzIFQuIiwiZmFtaWx5IjoiRnJhbXN0ZWQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU3RldmVuIiwiZmFtaWx5IjoiU2Fkcm8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiR2VvZmZyZXkiLCJmYW1pbHkiOiJTY2hsYWRvdyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBbmdlbGEgRGUiLCJmYW1pbHkiOiJQYWxtYeKAkERvdyJ9fV0sImlkcyI6eyJpc3NuIjpbIjEwMTUtMTYyMSIsIjE0MjAtOTA1NSJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMTAwNy9zMDAwMjctMDI0LTAxMDQ1LTIiLCJvcGVuYWxleCI6Ilc0MzkyNzEzMzk5IiwiZXhhIjoiaHR0cHM6Ly9saW5rLnNwcmluZ2VyLmNvbS9hcnRpY2xlLzEwLjEwMDcvczAwMDI3LTAyNC0wMTA0NS0yP2Vycm9yPWNvb2tpZXNfbm90X3N1cHBvcnRlZCZjb2RlPTgzY2Y4ZDg4LWM5YTAtNGNkZC04MjU2LTFhODVlNmFiZGFjMyJ9LCJ0aXRsZSI6IkludGVybmFsIHBob3NwaG9ydXMgbG9hZGluZyBhbHRlcnMgbnV0cmllbnQgbGltaXRhdGlvbiBhbmQgY29udHJpYnV0ZXMgdG8gY3lhbm9iYWN0ZXJpYWwgYmxvb21zIGluIGEgcG9seW1pY3RpYyBsYWtlIiwiYWJzdHJhY3QiOiJBYnN0cmFjdCBDbGVhciBMYWtlLCBhIG1lZGl1bS1zaXplZCBoeXBlcmV1dHJvcGhpYywgcG9seW1pY3RpYyBsYWtlIGluIG5vcnRoZXJuIENhbGlmb3JuaWEsIGhhcyBoYWQgcmVjdXJyaW5nIGhhcm1mdWwgY3lhbm9iYWN0ZXJpYSBibG9vbXMgKEhDQnMpIGZvciBvdmVyIGEgY2VudHVyeSBkZXNwaXRlIHJlZHVjdGlvbnMgaW4gZXh0ZXJuYWwgcGhvc3Bob3J1cyAoUCkgbG9hZGluZ3MuIEludGVybmFsIFAgbG9hZGluZyBjYW4gYWx0ZXIgbnV0cmllbnQgYXZhaWxhYmlsaXR5IGFuZCBsaW1pdGF0aW9uIHN1cHBvcnRpbmcgSENCcyBidXQgaXMgcmFyZWx5IHF1YW50aWZpZWQgb3IgY29tcGFyZWQgd2l0aCBleHRlcm5hbCBsb2Fkcy4gV2UgaGF2ZSBxdWFudGlmaWVkIGV4dGVybmFsIFAgbG9hZHMgZnJvbSAyMDE5IHRvIDIwMjEgZm9yIHRoZSB0aHJlZSBtYWluIHRyaWJ1dGFyaWVzIChhY2NvdW50aW5nIGZvciA0NiUgb2YgdGhlIGZsb3cpIGFuZCBpbnRlcm5hbCBsb2FkaW5ncyB1c2luZyB0d28gbWV0aG9kczogYSBQIG1hc3MgYmFsYW5jZSBhbmQgbW9kZWxlZCByZWxlYXNlIHJhdGVzIG9mIHNvbHVibGUgcmVhY3RpdmUgcGhvc3Bob3J1cyBmcm9tIG94aWMgYW5kIGFub3hpYyBzZWRpbWVudHMuIEluIGFkZGl0aW9uLCB3ZSBjb21iaW5lZCBoaWdoLWZyZXF1ZW5jeSBpbiBzaXR1IG1lYXN1cmVtZW50cyBvZiB3YXRlciB0ZW1wZXJhdHVyZSBhbmQgZGlzc29sdmVkIG94eWdlbiwgZGlzY3JldGUgZ3JhYiBzYW1wbGluZyBmb3IgbnV0cmllbnQgY2hlbWlzdHJ5LCBhbmQgcmVtb3RlIHNlbnNpbmcgdG8gZXhwbG9yZSB0aGUgcG90ZW50aWFsIGRyaXZlcnMgb2YgdGhlIG9ic2VydmVkIHZhcmlhYmlsaXR5IGFuZCBwcm92aWRlIGEgY29tcHJlaGVuc2l2ZSB2aWV3IG9mIHRoZSBzcGF0aW90ZW1wb3JhbCBkeW5hbWljcyBvZiBIQ0JzLiBCeSB1bmRlcnN0YW5kaW5nIHRoZSByZWxhdGl2ZSBjb250cmlidXRpb24gb2YgZXh0ZXJuYWwgYW5kIGludGVybmFsIG51dHJpZW50IGxvYWRpbmdzIGFuZCB0aGUgcmVsYXRpb25zaGlwIGJldHdlZW4gZW52aXJvbm1lbnRhbCBwYXJhbWV0ZXJzIGFuZCBIQ0JzLCBpbnRlcmFubnVhbCBibG9vbSB2YXJpYWJpbGl0eSBjYW4gYmUgYmV0dGVyIHByZWRpY3RlZC4gQ29tcGFyYXRpdmUgZXN0aW1hdGVzIG9mIGV4dGVybmFsIGFuZCBpbnRlcm5hbCBwaG9zcGhvcnVzIGxvYWRpbmcgaW5kaWNhdGUgdGhhdCBpbnRlcm5hbCBzb3VyY2VzIGFjY291bnRlZCBmb3IgNzDigJM5NSUgb2YgdGhlIHRvdGFsIFAgaW5wdXQgaW50byB0aGUgc3lzdGVtIGR1cmluZyB0aGUgc3R1ZHkgcGVyaW9kLiBDb250cmFyeSB0byBvdGhlciBsYWtlcywgdGhlIGludGVuc2l0eSBvZiB0aGUgc3VtbWVyIGJsb29tIHNlYXNvbiB3YXMgY29ycmVsYXRlZCB0byB0aGUgdGltaW5nIGFuZCBkdXJhdGlvbiBvZiBhbm94aWEgcmF0aGVyIHRoYW4gdGhlIG1hZ25pdHVkZSBvZiBzcHJpbmcgcnVub2ZmLiBJbnRlcm5hbGx5IHJlbGVhc2VkIFAgc2hpZnRlZCB0aGUgc3lzdGVtIGZyb20gcGhvc3Bob3J1cyB0byBuaXRyb2dlbiBsaW1pdGF0aW9uIGR1cmluZyB0aGUgc3VtbWVyLCBwb3RlbnRpYWxseSBmYXZvcmluZyB0aGUgcHJvbGlmZXJhdGlvbiBvZiBuaXRyb2dlbi1maXhpbmcgY3lhbm9iYWN0ZXJpYS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAwNy9zMDAwMjctMDI0LTAxMDQ1LTIiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vbGluay5zcHJpbmdlci5jb20vY29udGVudC9wZGYvMTAuMTAwNy9zMDAwMjctMDI0LTAxMDQ1LTIucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjQtMDMtMTIiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTExIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJBcXVhdGljIFNjaWVuY2VzIiwiaXNzdWUiOjIsInZvbHVtZSI6ODYsImltcGFjdEZhY3RvciI6IjIuODQifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiU3ByaW5nZXIgU2NpZW5jZStCdXNpbmVzcyBNZWRpYSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NzUsImNpdGVkQnlDb3VudCI6MjF9fSwicXVvdGUiOiJDb250cmFyeSB0byBvdGhlciBsYWtlcywgdGhlIGludGVuc2l0eSBvZiB0aGUgc3VtbWVyIGJsb29tIHNlYXNvbiB3YXMgY29ycmVsYXRlZCB0byB0aGUgdGltaW5nIGFuZCBkdXJhdGlvbiBvZiBhbm94aWEgcmF0aGVyIHRoYW4gdGhlIG1hZ25pdHVkZSBvZiBzcHJpbmcgcnVub2ZmLiBJbnRlcm5hbGx5IHJlbGVhc2VkIFAgc2hpZnRlZCB0aGUgc3lzdGVtIGZyb20gcGhvc3Bob3J1cyB0byBuaXRyb2dlbiBsaW1pdGF0aW9uIGR1cmluZyB0aGUgc3VtbWVyLCBwb3RlbnRpYWxseSBmYXZvcmluZyB0aGUgcHJvbGlmZXJhdGlvbiBvZiBuaXRyb2dlbi1maXhpbmcgY3lhbm9iYWN0ZXJpYS4gLSBbQ2FsY2l1bSBhbmQgcGhvc3BoYXRlIG1ldGFib2xpYyBkaXNvcmRlcnNdKGh0dHBzOi8vbGluay5zcHJpbmdlci5jb20vc3ViamVjdHMvY2FsY2l1bS1hbmQtcGhvc3BoYXRlLW1ldGFib2xpYy1kaXNvcmRlcnMpIEV1dHJvcGhpY2F0aW9uIHBvc2VzIGEgZ3JhdmUgdGhyZWF0IHRvIGxlbnRpYyBlY29zeXN0ZW1zIGdsb2JhbGx5LCBwcm9tb3RpbmcgdGhlIHByb2xpZmVyYXRpb24gb2YgaGFybWZ1bCBjeWFub2JhY3RlcmlhIGJsb29tcyAoSENCcykgdGhhdCByZWR1Y2UgYXF1YXRpYyBiaW9kaXZlcnNpdHksIHlpZWxkaW5nIGluY3JlYXNlZCB3YXRlciB0cmVhdG1lbnQgY29zdHMsIGFuZCB0aHJlYXRlbmluZyBodW1hbiBoZWFsdGggdGhyb3VnaCBibG9vbS1wcm9kdWNlZCBjeWFub3RveGlucyAoQXLDoW96IGV0IGFsLiJ9XQ%3D%3D.

The decay phase of HABs unleashes the most catastrophic oxygen drawdown. As cyanobacteria senesce and sink, bacterial decomposition of organic matter surges, exerting biochemical oxygen demand that consumes ambient DO at rates exceeding 1 mg/L per hour under anoxic conditions(Haxthausen, 2021; Sitote & Gebremedhine, 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllhYnNyYSBNZWxhayIsImZhbWlseSI6IlNpdG90ZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNaWNoYWVsIEdpcmltYXkiLCJmYW1pbHkiOiJHZWJyZW1lZGhpbmUifX1dLCJpZHMiOnsiaXNzbiI6W10sImlzYm4iOltdLCJkb2kiOiIxMC4yMDk0NC9wcmVwcmludHMyMDI0MDkuMDcwNC52MSIsIm9wZW5hbGV4IjoiVzQ0MDI0NjU0MTQiLCJleGEiOiJodHRwczovL3d3dy5wcmVwcmludHMub3JnL21hbnVzY3JpcHQvMjAyNDA5LjA3MDQifSwidGl0bGUiOiJDb21wcmVoZW5zaXZlIFJldmlldyBvZiBFdXRyb3BoaWNhdGlvbiBpbiBGcmVzaHdhdGVyIEVjb3N5c3RlbXM6IENhdXNlcywgRWZmZWN0cywgQXNzZXNzbWVudCwgYW5kIE1hbmFnZW1lbnQgU3RyYXRlZ2llcyIsImFic3RyYWN0IjoiVGhpcyByZXZpZXcgZXhhbWluZXMgZXV0cm9waGljYXRpb24gaW4gZnJlc2h3YXRlciBlY29zeXN0ZW1zLCBhIHdpZGVzcHJlYWQgZW52aXJvbm1lbnRhbCBpc3N1ZSBzdGVtbWluZyBmcm9tIGV4Y2Vzc2l2ZSBudXRyaWVudCBpbnB1dCwgbGFyZ2VseSBkdWUgdG8gaHVtYW4gYWN0aXZpdGllcy4gVGhlIHN0dWR5IGV4cGxvcmVzIHRoZSBvcmlnaW5zLCBjb25zZXF1ZW5jZXMsIGV2YWx1YXRpb24gdGVjaG5pcXVlcywgYW5kIGNvbnRyb2wgbWV0aG9kcyBmb3IgZXV0cm9waGljYXRpb24uIFByaW1hcnkgY2F1c2VzIGluY2x1ZGUgYWdyaWN1bHR1cmFsIGRpc2NoYXJnZSwgdXJiYW4gZGV2ZWxvcG1lbnQsIGFuZCBjbGltYXRlIHNoaWZ0cy4gVGhlIGVmZmVjdHMgYXJlIGZhci1yZWFjaGluZywgaW1wYWN0aW5nIGJpb2RpdmVyc2l0eSwgd2F0ZXIgcXVhbGl0eSwgYW5kIHNvY2lvLWVjb25vbWljIGZhY3RvcnMuIFRoZSBhc3Nlc3NtZW50IHV0aWxpemVzIGEgY29tYmluYXRpb24gb2YgY2hlbWljYWwsIHBoeXNpY2FsLCBhbmQgYmlvbG9naWNhbCBtYXJrZXJzLCB3aXRoIGN1dHRpbmctZWRnZSB0ZWNobm9sb2dpZXMgbGlrZSByZW1vdGUgc2Vuc2luZyBpbXByb3ZpbmcgbW9uaXRvcmluZyBlZmZvcnRzLiBNaXRpZ2F0aW9uIHN0cmF0ZWdpZXMgYXJlIGRpdmVyc2UsIGluY29ycG9yYXRpbmcgcHJldmVudGF0aXZlIGFjdGlvbnMgKHN1Y2ggYXMgbnV0cmllbnQgbGltaXRhdGlvbiBhbmQgbGFuZCBtYW5hZ2VtZW50KSwgY29ycmVjdGl2ZSBtZWFzdXJlcyAoaW5jbHVkaW5nIGJpby1tYW5pcHVsYXRpb24gYW5kIGNoZW1pY2FsIGFwcGxpY2F0aW9ucyksIGFuZCByZWd1bGF0b3J5IGluaXRpYXRpdmVzIChsaWtlIHdhdGVyIHF1YWxpdHkgZ3VpZGVsaW5lcyBhbmQgbnV0cmllbnQgZXhjaGFuZ2Ugc2NoZW1lcykuIFdoaWxlIGNhc2Ugc3R1ZGllcyBkZW1vbnN0cmF0ZSBzdWNjZXNzZnVsIGludGVydmVudGlvbnMsIHRoZXkgYWxzbyByZXZlYWwgb25nb2luZyBjaGFsbGVuZ2VzLCBwYXJ0aWN1bGFybHkgZWNvc3lzdGVtIHVucHJlZGljdGFiaWxpdHkgYW5kIGNsaW1hdGUgY2hhbmdlIGluZmx1ZW5jZXMuIFRoZSByZXNlYXJjaCBoaWdobGlnaHRzIHRoZSBwcm9taXNlIG9mIGVtZXJnaW5nIHRlY2hub2xvZ2llcywgbm90YWJseSBtYWNoaW5lIGxlYXJuaW5nLCBpbiBlbmhhbmNpbmcgbWFuYWdlbWVudCBwcmFjdGljZXMuIEtub3dsZWRnZSBnYXBzIHBlcnNpc3QsIGVzcGVjaWFsbHkgcmVnYXJkaW5nIGV1dHJvcGhpY2F0aW9uLWJpb2RpdmVyc2l0eSBkeW5hbWljcyBpbiB0cm9waWNhbCBlbnZpcm9ubWVudHMgYW5kIHRoZSBzb2Npby1lY29ub21pYyByYW1pZmljYXRpb25zIG9mIGludGVydmVudGlvbnMuIFRoZSBzdHVkeSBlbXBoYXNpemVzIHRoZSBpbXBvcnRhbmNlIG9mIGZsZXhpYmxlLCBjb21wcmVoZW5zaXZlIGFwcHJvYWNoZXMgdGhhdCBibGVuZCBzY2llbnRpZmljIGluc2lnaHRzIHdpdGggcHJhY3RpY2FsIHNvbHV0aW9ucyBhbmQgc3Ryb25nIHBvbGljeSBzdHJ1Y3R1cmVzIHRvIHRhY2tsZSB0aGlzIGludHJpY2F0ZSBlbnZpcm9ubWVudGFsIGNoYWxsZW5nZSBpbiBhbiBlcmEgb2YgY2xpbWF0ZSBjaGFuZ2UuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjIwOTQ0L3ByZXByaW50czIwMjQwOS4wNzA0LnYxIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjA5NDQvcHJlcHJpbnRzMjAyNDA5LjA3MDQudjEiLCJpc3N1ZWREYXRlIjoiMjAyNC0wOS0xMCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTEiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IlByZXByaW50cy5vcmciLCJpbXBhY3RGYWN0b3IiOiIxLjE1In0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjowLCJjaXRlZEJ5Q291bnQiOjR9fSwicXVvdGUiOiJUaGUgc3R1ZHkgZW1waGFzaXplcyB0aGUgaW1wb3J0YW5jZSBvZiBmbGV4aWJsZSwgY29tcHJlaGVuc2l2ZSBhcHByb2FjaGVzIHRoYXQgYmxlbmQgc2NpZW50aWZpYyBpbnNpZ2h0cyB3aXRoIHByYWN0aWNhbCBzb2x1dGlvbnMgYW5kIHN0cm9uZyBwb2xpY3kgc3RydWN0dXJlcyB0byB0YWNrbGUgdGhpcyBpbnRyaWNhdGUgZW52aXJvbm1lbnRhbCBjaGFsbGVuZ2UgaW4gYW4gZXJhIG9mIGNsaW1hdGUgY2hhbmdlLiBFdXRyb3BoaWNhdGlvbiwgY2F1c2VkIGJ5IGV4Y2Vzc2l2ZSBudXRyaWVudCBlbnJpY2htZW50IGluIHdhdGVyIGJvZGllcywgaXMgYSBzaWduaWZpY2FudCBlbnZpcm9ubWVudGFsIGNoYWxsZW5nZSBmb3IgZnJlc2h3YXRlciBlY29zeXN0ZW1zIG9uIGEgZ2xvYmFsIHNjYWxlLiBQcmltYXJpbHkgZHJpdmVuIGJ5IG5pdHJvZ2VuIGFuZCBwaG9zcGhvcnVzIGlucHV0cywgdGhpcyBwcm9jZXNzIHJlc3VsdHMgaW4gdGhlIG92ZXJwcm9kdWN0aW9uIG9mIGFsZ2FlIGFuZCBhcXVhdGljIHBsYW50cywgbGVhZGluZyB0byBhIGNhc2NhZGUgb2YgZWNvbG9naWNhbCBkaXN0dXJiYW5jZXMsIGluY2x1ZGluZyBveHlnZW4gZGVwbGV0aW9uIChoeXBveGlhKSwgbG9zcyBvZiBiaW9kaXZlcnNpdHksIGFuZCBzZXZlcmUgd2F0ZXIgcXVhbGl0eSBkZWdyYWRhdGlvbiAoS2FrYWRlIGV0IGFsLiwgMjAyMTsgTWVlcmhvZmYgZXQgYWwuLCAyMDIyOyBXdXJ0c2JhdWdoIGV0IGFsLiwgMjAxOSkuIn0seyJvcmlnaW4iOjQsIndvcmsiOnsidHlwZSI6OCwic3VidHlwZSI6eyJvbmVvZktpbmQiOiJhcnRpY2xlVHlwZSIsImFydGljbGVUeXBlIjoxfSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiS2FybCBSYXNtdXMgQXVndXN0IiwiZmFtaWx5IjoiSGF4dGhhdXNlbiJ9fV0sImlkcyI6eyJpc3NuIjpbXSwiaXNibiI6W10sIm1hZyI6IjMxNzcxMTMyMDciLCJvcGVuYWxleCI6IlczMTc3MTEzMjA3In0sInRpdGxlIjoiU2VkaW1lbnQgbWljcm9iaWFsIGZ1ZWwgY2VsbHMgZm9yIGxha2UgcmVzdG9yYXRpb24iLCJhYnN0cmFjdCI6IkV1dHJvcGhpY2F0aW9uIG9mIGxha2VzIGlzIHViaXF1aXRvdXMsIGFzIDMwLTQwICUgb2YgdGhlIHdvcmxk4oCZcyBsYWtlcyBhcmUgZXV0cm9waGljLCBhbmQgY29uZGl0aW9ucyBjb250aW51b3VzbHkgZGV0ZXJpb3JhdGluZy4gRXV0cm9waGljYXRpb24gaXMgYSBjb25kaXRpb24gb2Ygb3ZlcmZlcnRpbGl6YXRpb24sIHdoZXJlIGFsbCByZXF1aXJlbWVudHMgZm9yIHJhcGlkIGdyb3d0aCBvZiBhbGdhZSBhcmUgbWV0LCByZXN1bHRpbmcgaW4gYWxnYWwgYmxvb21zLiBUaGlzIGNhdXNlcyBhIHNlbGYtc3VzdGFpbmVkLCB2aWNpb3VzIGN5Y2xlIHdoZXJlIHBvb3IgZWNvbG9naWNhbCBjb25kaXRpb25zIGFyZSBtYWludGFpbmVkIHVudGlsIG51dHJpZW50cyBhcmUgbG9zdCBmcm9tIHRoZSBsYWtlIG92ZXIgbWFueSB5ZWFycy4gTGFrZSByZXN0b3JhdGlvbiBzZWVrcyB0byBicmVhayB0aGlzIHZpY2lvdXMgY3ljbGUuIEluYWRlcXVhdGUgYXZhaWxhYmlsaXR5IG9mIGFueSBzaW5nbGUgbnV0cmllbnQgaW4gd2lsbCBsaW1pdCBhbGdhbCBncm93dGgsIHR5cGljYWxseSBwaG9zcGhvcnVzIGlzIHRhcmdldGVkLiBUaGUgc2NvcGUgb2YgdGhpcyBQaEQgdGhlc2lzIHdhcyB0byBkZXZlbG9wLCB0ZXN0IGFuZCBvcHRpbWl6ZSBzZWRpbWVudCBtaWNyb2JpYWwgZnVlbCBjZWxscyAoU01GQykgYXMgYSBub3ZlbCBtZXRob2QgZm9yIGxha2UgcmVzdG9yYXRpb24uIFRoZSB0aGVzaXMgc3BhbnMgdGhlIHR3byBtYWluIHRvcGljcyBvZiBTTUZDcyBhbmQgbGFrZSByZXN0b3JhdGlvbiB0ZWNobmlxdWVzLiBMYWtlIHJlc3RvcmF0aW9uIHRlY2huaXF1ZXMgYXJlIHJldmlld2VkLCBwcm92aWRpbmcgdW5kZXJzdGFuZGluZyBvZiB0aGUgdW5kZXJseWluZyBtZWNoYW5pc21zLiBTTUZDcyBhcmUgYXBwcm9hY2hlZCBmcm9tIGFuIGVuZ2luZWVyaW5nIHBlcnNwZWN0aXZlIGFuZCBhIGZvY3VzIG9uIGxha2UgcmVtZWRpYXRpb24uIFRoZSBzcGVjaWZpYyBvYmplY3RpdmVzIHdlcmUgdG8gdGVzdCBhIGxvdy1jb3N0IGVsZWN0cm9kZSBtYXRlcmlhbCBmb3IgaXRzIGR1cmFiaWxpdHksIHByYWN0aWNhbGl0eSwgYW5kIHBlcmZvcm1hbmNlLiBQZXJmb3JtYW5jZSB3YXMgYXNzZXNzZWQgYXMgdGhlIGFiaWxpdHkgdG8gbGltaXQgcGhvc3Bob3J1cyByZWxlYXNlIGZyb20gc2VkaW1lbnRzLCBpbmhpYml0IG1ldGhhbmUgcmVsZWFzZSwgYW5kIHRoZSBhZGRlZCBiZW5lZml0cyBvZiBkZWdyYWRpbmcgcG9sbHV0YW50cywgYW5kIGdlbmVyYXRpbmcgZWxlY3RyaWNpdHkuIFByb29mIG9mIGNvbmNlcHQgd2FzIHNob3duIGluIGxhYm9yYXRvcnkgcmVhY3RvcnMgKDEgTCksIHRoZSBlZmZlY3Qgb2YgdXBzY2FsaW5nIHRvIDIwMCBMIGluIG5hdHVyYWwgZW52aXJvbm1lbnQgd2FzIHN0dWRpZWQgaW4gb3V0ZG9vciBtZXNvY29zbXMsIGFuZCB0aGVuIHRlc3RlZCBhdCBwaWxvdCBzY2FsZSBpbiBhIGV1dHJvcGhpYyBmcmVzaHdhdGVyIGxha2UuIFBpbG90IGluc3RhbGxhdGlvbnMgd2VyZSB0aGUgbGFyZ2VzdCBpbiB0aGUgd29ybGQgb2YgdGhlaXIga2luZCBhbmQgd2VyZSBvcGVyYXRlZCBmb3IgMiB5ZWFycy4gU3RhaW5sZXNzIHN0ZWVsIHdhcyBzZWxlY3RlZCBmb3IgdGVzdGluZyBhcyBlbGVjdHJvZGUgbWF0ZXJpYWwgZHVlIHRvIGl0cyBsb3cgY29zdCwgZHVyYWJpbGl0eSwgYW5kIGF2YWlsYWJpbGl0eS4gUGFpcnMgb2YgZWxlY3Ryb2RlcyB3ZXJlIGVtYmVkZGVkIGluIHRoZSBzZWRpbWVudCAodGhpcyBpcyBjYWxsZWQgdGhlIGFub2RlKSBhbmQgc3VzcGVuZGVkIGluIHRoZSBvdmVybHlpbmcgd2F0ZXIgKHRoaXMgaXMgY2FsbGVkIHRoZSBjYXRob2RlKS4gVGhlIHR3byBlbGVjdHJvZGVzIHdlcmUgY29ubmVjdGVkIGFjcm9zcyBhbiBleHRlcm5hbCByZXNpc3RvciB3aXRoIGVsZWN0cmljYWwgd2lyZXMuIE1pY3JvYmVzIGluIHRoZSBzZWRpbWVudCB1c2UgdGhlIGFub2RlIHBhaXIgYXMgZWxlY3Ryb24gYWNjZXB0b3IsIGVuYWJsaW5nIHJlc3BpcmF0aW9uIGluIHRoZSBhbm94aWMgc2VkaW1lbnQuIEVsZWN0cm9ucyB0cmF2ZWwgZnJvbSB0aGUgc2VkaW1lbnQgdmlhIHRoZSBlbGVjdHJpY2FsIGNvbm5lY3RvciB0byB0aGUgb3Zlcmx5aW5nIGNhdGhvZGUgd2hlcmUgb3h5Z2VuIGluIHRoZSB3YXRlciBpcyB0aGUgdGVybWluYWwgZWxlY3Ryb24gYWNjZXB0b3IgY29tcGxldGluZyB0aGUgY2lyY3VpdC4gQXMgYW4gYWRkZWQgYmVuZWZpdCwgZGVncmFkYXRpb24gb2YgcG9sbHV0YW50cyB1c2luZyBTTUZDIHdhcyB0ZXN0ZWQgaW4gbWFyaW5lIGhhcmJvdXIgc2VkaW1lbnRzLCBidXQgbm8gY29uY2x1c2l2ZSByZXN1bHRzIG9idGFpbmVkLiBMYWJvcmF0b3J5IHJlYWN0b3JzIHdlcmUgb3BlcmF0ZWQgZm9yIG1vcmUgdGhhbiAyMDAgZGF5cyB1c2luZyBwZXRyb2xldW0gYXMgc29sZSBjYXJib24gc291cmNlLCBjb25maXJtaW5nIG9pbCBkZWdyYWRhdGlvbiBieSBTTUZDcy4gQSByZW1vdmFsIHJhdGUgb2YgMC45IGcgcGV0cm9sZXVtIG0tMyBkLTEgcmVhY3RvciB2b2x1bWUgd2FzIGVzdGltYXRlZCBmcm9tIG1lYXN1cmVkIGVsZWN0cmljYWwgb3V0cHV0LiBUaGlzIHdhcyBkb25lIGJ5IGFzc3VtcHRpb24gb2YgdGhlIGNoZW1pY2FsIHJlYWN0aW9uIG9jY3VycmluZywgZnJvbSB0aGlzIHRoZSBudW1iZXIgb2YgZWxlY3Ryb25zIHRyYW5zZmVycmVkIGZyb20gb2lsIHRvIHdhdGVyIHdhcyBrbm93biBhbmQgY291bGQgYmUgY29ycmVsYXRlZCB0byB0aGUgbWVhc3VyZWQgZmxvdyBvZiBlbGVjdHJvbnMgKGN1cnJlbnQpIHRocm91Z2ggdGhlIGV4dGVybmFsIGNpcmN1aXQuIFN0YWlubGVzcyBzdGVlbCBlbGVjdHJvZGVzIHdlcmUgb3BlcmF0ZWQgaW4gZnJlc2h3YXRlciBsYWtlIGVudmlyb25tZW50cyBmb3IgbW9yZSB0aGFuIDQgeWVhcnMgd2l0aCBtaW5pbWFsIGRhbWFnZSB0byB0aGUgbWF0ZXJpYWwuIFRoZSBlbGVjdHJvZGUgaXMgdGhlIG1haW4gbWF0ZXJpYWwgY29zdCBpbiBzdWNoIHN5c3RlbXMuIEhvd2V2ZXIsIEFJU0kzMTYgc3RhaW5sZXNzIHN0ZWVsIHNpbWlsYXIgdG8gd2hhdCB3YXMgdXNlZCBpbiB0aGlzIHRoZXNpcyB3YXMgYXQgdGhlIHRpbWUgb2Ygd3JpdGluZyBhdmFpbGFibGUgYXQgYnVsayBjb3N0IG9mIDAuMDkxIOKCrCBtLTIsIGNvcnJlc3BvbmRpbmcgdG8gOTEwIOKCrCBhY3JlLTEsIHNpZ25pZmljYW50bHkgbG93ZXIgdGhhbiBvdGhlciBjb21tb24gbWF0ZXJpYWxzLiBDYXRob2RlcyBhdCBsYXJnZSBzY2FsZSBjb3VsZCBiZSBtb3ZlZCBob3Jpem9udGFsbHkgYW5kIHZlcnRpY2FsbHkgd2l0aCBtaW5vciBpbXBhY3QgdG8gcGVyZm9ybWFuY2UuIFBpbG90IHNjYWxlIFNNRkNzIGdlbmVyYXRlZCAwLjM4MyBtVyBtLTIgc3VzdGFpbmVkIGFuZCAyNy40IG1XIG0tMiBwZWFrIG91dHB1dC4gVGhlIHN5c3RlbSB3YXMgbGltaXRlZCBieSB0aGUgY2F0aG9kZSBzdXJmYWNlIGFyZWEsIG1lYW5pbmcgbXVsdGktbGF5ZXIgY2F0aG9kZXMgc2hvdWxkIGJlIHRlc3RlZCBpbiB0aGUgZnV0dXJlIGlmIGVsZWN0cmljYWwgcG93ZXIgcHJvZHVjdGlvbiBpcyB0aGUgZ29hbC4gVGhlIGNhdGhvZGUgc2hvdWxkIGJlIHBsYWNlZCBhdCBhIGRlcHRoIHRoYXQgZW5zdXJlcyBoaWdoIGRpc3NvbHZlZCBveHlnZW4gY29uY2VudHJhdGlvbnMgdG8gYXZvaWQgdGhlIHN5c3RlbSB0byBiZSBsaW1pdGVkLiBIb3dldmVyLCB0b28gaGlnaCBlbGVjdHJpY2l0eSBnZW5lcmF0aW9uIHdhcyBmb3VuZCBjb3VudGVycHJvZHVjdGl2ZSBmb3IgcmVtZWRpYXRpb24gcHVycG9zZXMgYmVjYXVzZSB0aGlzIHdvdWxkIGluY3JlYXNlIHNlZGltZW50IHR1cm5vdmVyIHJhdGUsIHJlbGVhc2luZyBwb2xsdXRhbnRzIHN0b3JlZCBpbiB0aGUgc2VkaW1lbnRzLiBQIGJpbmRpbmcgd2FzIG5vdCBmb3VuZCB0byByZXF1aXJlIGhpZ2ggdHVybm92ZXIgb2Ygc2VkaW1lbnQgb3JnYW5pYyBtYXR0ZXIuIFRoZSB0dXJub3ZlciBjYW4gYmUgY29udHJvbGxlZCBieSBjaGFuZ2luZyB0aGUgZXh0ZXJuYWwgcmVzaXN0YW5jZS4gSGlnaCByZXNpc3RhbmNlIGxpbWl0cyB0aGUgZWxlY3RyaWNhbCBwZXJmb3JtYW5jZSwgbG93IHJlc2lzdGFuY2UgaW5jcmVhc2VzIHNlZGltZW50IHR1cm5vdmVyIGFuZCByaXNrcyBkZXN0YWJpbGl6aW5nIHRoZSBzeXN0ZW0uIFRvIGFjaGlldmUgcmVkdWNlZCBjb25jZW50cmF0aW9ucyBvZiBwaG9zcGhhdGUgaW4gdGhlIHdhdGVyIGNvbHVtbiwgYW5vZGVzIG11c3QgYmUgcGxhY2VkIGluIHRoZSBzZWRpbWVudCBzdXJmYWNlIHNpbmNlIGEgZmV3IG1pbGxpbWV0cmVzIG9mIHNlZGltZW50IG1heSBjb250YWluIGFuZCByZWxlYXNlIGVub3VnaCBQIHRvIHN1c3RhaW4gZXV0cm9waGljYXRpb24uIFBob3NwaGF0ZSB3YXMgbWVhc3VyZWQgaW4gdGhlIHdhdGVyIGNvbHVtbiBuZWFyIHRoZSBzZWRpbWVudCwgaW4gc3lzdGVtcyB3aXRoIGFuZCB3aXRob3V0IGVsZWN0cm9kZXMuIFBob3NwaGF0ZSBjb25jZW50cmF0aW9ucyB3ZXJlIHN1Y2Nlc3NmdWxseSByZWR1Y2VkIGJ5IHVwIHRvIDk1ICUgaW4gMi1jaGFtYmVyIGxhYm9yYXRvcnkgcmVhY3RvcnMsIGFuZCBieSB1cCB0byA5OCAlIGluIHRoZSBwaWxvdCBpbnN0YWxsYXRpb24uIEluIHRoZSBwaWxvdCBleHBlcmltZW50LCBwaG9zcGhhdGUgY29uY2VudHJhdGlvbnMgYWJvdmUgZWxlY3Ryb2RlcyB3ZXJlIGxvd2VyIGluIGFsbCBtZWFzdXJlbWVudHMsIGNvbXBhcmVkIHRvIGNvbnRyb2xzLiBTZWRpbWVudCByZWRveCBwb3RlbnRpYWxzIGluY3JlYXNlZCBmcm9tIC00MDAgbVYgdnMgc3RhbmRhcmQgaHlkcm9nZW4gZWxlY3Ryb2RlIChTSEUpIHRvIC0xNzUgbVYgdnMgU0hFLCBjb3JyZXNwb25kaW5nIHRvIGEgbW92ZSBmcm9tIGEgbWV0aGFub2dlbmV0aWMgZG9tYWluIHRvIHN1bHBodXIgcmVkdWNpbmcuIEFuYWx5c2lzIG9mIHRoZSBtaWNyb2JpYWwgY29tbXVuaXR5IGluZGljYXRlZCBlbmhhbmNlZCBzdWxwaHVyIGN5Y2xpbmcgd2hlcmUgZWxlY3Ryb2RlcyBveGlkaXplIHN1bHBoaWRlIHdoaWNoIHRoZSBiYWN0ZXJpYSBhZ2FpbiByZWR1Y2UgdG8gb2J0YWluIGVuZXJneS4gSXQgd2FzIGh5cG90aGVzaXNlZCB0aGF0IFNNRkMgcHJvbW90ZSBoaWdoZXIgcmVkb3ggcG90ZW50aWFscyBieSBmYWNpbGl0YXRpbmcgb3hpZGF0aW9uIG9mIHN1bHBodXIgYW5kIGlyb24gc3BlY2llcywgaW5oaWJpdGluZyBIMlMgZm9ybWF0aW9uIGFuZCBGZSBzY2F2ZW5naW5nIGJ5IEZlUyBmb3JtYXRpb24sIHRodXMgYWxsb3dpbmcgZm9yIFAgYmluZGluZy4gQSBsb3dlciByZWxhdGl2ZSBhYnVuZGFuY2Ugb2YgbWV0aGFub2dlbnMgb24gZWxlY3Ryb2RlcyBjb21wYXJlZCB0byBjb250cm9scyBzdWdnZXN0cyBzdXBwcmVzc2lvbiBvciBvdXRjb21wZXRpbmcgb2YgbWV0aGFub2dlbnMuUXVhbnRpZmljYXRpb24gb2YgbWV0aGFuZSBwcm9kdWN0aW9uIHdhcyBkb25lIG9uIHRoZSBiYXNpcyBvZiBhIENINDpDTzIgcmF0aW8gdG8gYWxsb3cgY29tcGFyYWJpbGl0eSBiZXR3ZWVuIHN5c3RlbXMuIE1ldGhhbmUgZW1pc3Npb24gd2FzIHJlZHVjZWQgYnkgdXAgdG8gOTkgJSBhbmQgbWFpbnRhaW5lZCBpbmhpYml0aW9uIGZvciBzZXZlcmFsIG1vbnRocyBpbiBsYWJvcmF0b3J5IHN5c3RlbXMgd2l0aCBlbGVjdHJvZGVzLiBJbiB0aGUgcGlsb3QgaW5zdGFsbGF0aW9ucyBubyBnYXMgc2FtcGxlcyBjb3VsZCBiZSBjb2xsZWN0ZWQsIGFuZCBubyBDTzI6Q0g0IHJhdGlvcyB3ZXJlIG9idGFpbmVkLiBBZXJhdGlvbiBpcyB0aGUgY3VycmVudCByZXN0b3JhdGlvbiB0ZWNobmlxdWUgdGhhdCBtb3N0IGNsb3NlbHkgcmVzZW1ibGVzIHRoZSBwcm9wb3NlZCBTTUZDIHRlY2huaXF1ZS4gQWVyYXRpb24gaGFzIHByZXZpb3VzbHkgZmFpbGVkIHRvIHJlZHVjZSBQIHJlbGVhc2UgaW4gc2V2ZXJhbCBzZXBhcmF0ZSBpbnZlc3RpZ2F0aW9ucy4gU01GQ3MgYXMgYSByZW1lZGlhdGlvbiB0b29sIHdhcyBmb3VuZCB0byBvZmZlciBzZXZlcmFsIGJlbmVmaXRzIG92ZXIgY29udmVudGlvbmFsIG1ldGhvZHM6IFNNRkNzIG9mZmVycyBjb250aW51b3VzIGVmZmVjdCwgd2hpbGUgb3RoZXIgdGVjaG5pcXVlcyByZXF1aXJlIGNvbnRpbnVvdXMgb3IgcmVwZWF0ZWQgdHJlYXRtZW50LiBTTUZDcyBjYW4gYmUgY29udHJvbGxlZCBpbiByZWFsLXRpbWUgYnkgYWx0ZXJpbmcgdGhlIGV4dGVybmFsIHJlc2lzdG9yIGFuZCBjYW4gYmUgZWFzaWx5IG1vbml0b3JlZCBieSBtZWFzdXJpbmcgdGhlIHByb2R1Y2VkIGVsZWN0cmljYWwgb3V0cHV0LiBJZiBpbXBsZW1lbnRlZCBhdCBsYXJnZSBzY2FsZSBpdCBpcyBleHBlY3RlZCB0aGF0IHRoZSBlbGVjdHJvZGUgZmllbGRzIHdpbGwgaGVscCBtYWludGFpbiBsb3cgcGhvc3BoYXRlIGNvbmNlbnRyYXRpb25zIGluIHRoZSB3YXRlciBmb3IgZGVjYWRlcyB3aXRoIG1pbmltYWwgaW50ZXJ2ZW50aW9uLCBhbmQgYWZ0ZXIgc29tZSB0aW1lLCBiZW50aGljIHBsYW50cyBhcmUgZXhwZWN0ZWQgdG8gcmV0dXJuLCB0aHVzIG1haW50YWluaW5nIHRoZSBiYWxhbmNlIGhlbmNlIGZvcnRoLiIsInVybCI6Imh0dHBzOi8vbG9jYWwuZm9yc2tuaW5nc3BvcnRhbC5kay9sb2NhbC9ka2ktY2dpL3dzL2NyaXMtbGluaz9zcmM9ZHR1JmlkPWR0dS1kNjZkMzM3MC0yNTZmLTQ4ODUtYTA5ZS0zMzZhOTI0ZjRiZWQmdGk9U2VkaW1lbnQlMjBtaWNyb2JpYWwlMjBmdWVsJTIwY2VsbHMlMjBmb3IlMjBsYWtlJTIwcmVzdG9yYXRpb24iLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vYmFja2VuZC5vcmJpdC5kdHUuZGsvd3MvZmlsZXMvMjQ4MDY5NTg5L1RoZXNpc19vbmxpbmVfdmVyc2lvbl9LYXJsX0hheHRoYXVzZW4ucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjEtMDEtMDEiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA4IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJSZXNlYXJjaCBQb3J0YWwgRGVubWFyayIsImltcGFjdEZhY3RvciI6IjAuMDQifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjo3Nn19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJUZWNobmljYWwgVW5pdmVyc2l0eSBvZiBEZW5tYXJrIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjowLCJjaXRlZEJ5Q291bnQiOjF9fSwicXVvdGUiOiJFeHRlcm5hbCBpbnB1dHMgaW5jcmVhc2UgUCBjb25jZW50cmF0aW9uIGluICB0aGUgd2F0ZXIsIGNhdXNpbmcgYWxnYWwgYmxvb21zIHRoYXQgc2hhZG93IHRoZSBsYWtlIGJvdHRvbSwgYW5kIGJlbnRoaWMgbWFjLXJvcGh5dGVzIHJlY2VkZS4gUHJlZGF0b3J5IGZpc2ggYXJlIHVuYWJsZSB0byBodW50LCBhbmQgdGhlaXIgZm9vZCBzb3VyY2UgaXMgIGRpc3J1cHRlZCBzdWNoIGFzIGZpc2ggZmVlZGluZyBvbiBiZW50aGljIGludmVydGVicmF0ZXMgYW5kIGJvdHRvbS1kd2VsbGluZyBhbmltYWxzIGFyZSBzdWZmb2NhdGVkIChWYW5kZXIgWmFuZGVuIGFuZCBWYWRlYm9uY29ldXIgMjAwMiksIHNvIHRoYXQgcGxhbmt0aXZvcm91cyBmaXNoIHNwZWNpZXMgZG9taW5hdGUuIFRoaXMgaW5jcmVhc2VzIHByZWRhdGlvbiBvbiB6b28gcGxhbmt0b24gIGFuZCByZWR1Y2VzIGdyYXppbmcgb24gYWxnYWUuIEFsZ2FlIHByb2xpZmVyYXRlLCBkaWUsIGFuZCBzaW5rIHRvIHRoZSBib3R0b20sICBjb25zdW1pbmcgb3h5Z2VuIHdoZW4gZGVncmFkZWQsIHdoaWNoIGxlYWRzIHRvIG94eWdlbiBkZWZpY2l0IGluIHRoZSBib3QtdG9tLCBraWxsaW5nIGZpc2ggKFd1cnRzYmF1Z2ggZXQgYWwuIDIwMTkpLiBGaWd1cmUgMjogVmljaW91cyBjeWNsZXMgb2YgZXV0cm9waGljYXRpb24uIExlZnQ6IEludGVybGlua2VkIHByb2Nlc3NlcyBzdXN0YWluaW5nIGV1dHJvcGhpLWNhdGlvbiBpbiBhIGxha2UuIEJsdWU6IHByb2Nlc3NlcyBpbiB3YXRlciwgZ3JlZW46IHByb2Nlc3NlcyBpbnZvbHZpbmcgbWljcm9hbGdhZSwgZ3JleTogc2VkLWltZW50IHJlbGF0ZWQgcHJvY2Vzc2VzLCB5ZWxsb3c6IHByb2Nlc3NlcyBpbnZvbHZpbmcgZmF1bmEuIFJpZ2h0LCBkYXNoZWQgbGluZTogVGhlIG9yZGVyIGluIHdoaWNoIGVsZWN0cm9uIGFjY2VwdG9ycyBhcmUgY29uc3VtZWQgaW4gdGhlIGFic2VuY2Ugb2YgTzIgc2hvd24gYXMgY29uY2VudHJhdGlvbiB2ZXJzdXMgIGRlcHRoIGluIHNlZGltZW50LCBhbmQgYXQgd2hpY2ggcG90ZW50aWFsIHRoaXMgb2NjdXJzIChCZXJuZXIgMTk4MSwgYnkgY291cnRlc3kgb2YgdGhlIHNvY2lldHkgIGZvciBzZWRpbWVudGFyeSBnZW9sb2d5KS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjE3LCJiYm94Ijp7InRvcCI6NzkuOTMzMTUxMjQ1MTE3MTksImxlZnQiOjE0LjI5NTUwMzYxNjMzMzAwOCwid2lkdGgiOjcxLjkzODkzNDMyNjE3MTg4LCJoZWlnaHQiOjIuMDUzNDc3Mjg3MjkyNDgwNX19LHsicGFnZU51bWJlciI6MTcsImJib3giOnsidG9wIjo4MS45NzEzNDM5OTQxNDA2MiwibGVmdCI6MTQuMjk1NTAzNjE2MzMzMDA4LCJ3aWR0aCI6NzEuMzMzMDAwMTgzMTA1NDcsImhlaWdodCI6Mi4wNTM0NzcyODcyOTI0ODA1fX0seyJwYWdlTnVtYmVyIjoxNywiYmJveCI6eyJ0b3AiOjg0LjAyMzc5NjA4MTU0Mjk3LCJsZWZ0IjoxNC4yOTU1MDM2MTYzMzMwMDgsIndpZHRoIjo3MS44MTIwMTkzNDgxNDQ1MywiaGVpZ2h0IjoyLjA1MzQ3NzI4NzI5MjQ4MDV9fSx7InBhZ2VOdW1iZXIiOjE3LCJiYm94Ijp7InRvcCI6ODYuMDYxOTk2NDU5OTYwOTQsImxlZnQiOjE0LjI5NTUwMzYxNjMzMzAwOCwid2lkdGgiOjcxLjk1NzM1OTMxMzk2NDg0LCJoZWlnaHQiOjIuMDUzNDc3Mjg3MjkyNDgwNX19LHsicGFnZU51bWJlciI6MTgsImJib3giOnsidG9wIjo5LjkyNjQ4NzkyMjY2ODQ1NywibGVmdCI6MTQuMjk1NTAzNjE2MzMzMDA4LCJ3aWR0aCI6NzEuOTc0MDYwMDU4NTkzNzUsImhlaWdodCI6MTAuMjM0Nzc3NDUwNTYxNTIzfX0seyJwYWdlTnVtYmVyIjoxOCwiYmJveCI6eyJ0b3AiOjQ0LjczNDA0MzEyMTMzNzg5LCJsZWZ0IjoxNC4yOTU1MDM2MTYzMzMwMDgsIndpZHRoIjo3MS44OTM0MzI2MTcxODc1LCJoZWlnaHQiOjEwLjQwMzI4OTc5NDkyMTg3NX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjkzIn19XQ%3D%3D. In eutrophic systems, this process generates hypoxic bottom layers (<2 mg/L DO), solubilizing iron-bound phosphorus and ammonia, perpetuating a vicious feedback loop (Swann et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1pY2FoIE0uIiwiZmFtaWx5IjoiU3dhbm4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQWxpY2lhIiwiZmFtaWx5IjoiQ29ydMOpcyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBbGV4YW5kZXIgTC4iLCJmYW1pbHkiOiJGb3JyZXN0In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5pY2hvbGFzIFQuIiwiZmFtaWx5IjoiRnJhbXN0ZWQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU3RldmVuIiwiZmFtaWx5IjoiU2Fkcm8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiR2VvZmZyZXkiLCJmYW1pbHkiOiJTY2hsYWRvdyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBbmdlbGEgRGUiLCJmYW1pbHkiOiJQYWxtYeKAkERvdyJ9fV0sImlkcyI6eyJpc3NuIjpbIjEwMTUtMTYyMSIsIjE0MjAtOTA1NSJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMTAwNy9zMDAwMjctMDI0LTAxMDQ1LTIiLCJvcGVuYWxleCI6Ilc0MzkyNzEzMzk5IiwiZXhhIjoiaHR0cHM6Ly9saW5rLnNwcmluZ2VyLmNvbS9hcnRpY2xlLzEwLjEwMDcvczAwMDI3LTAyNC0wMTA0NS0yP2Vycm9yPWNvb2tpZXNfbm90X3N1cHBvcnRlZCZjb2RlPTgzY2Y4ZDg4LWM5YTAtNGNkZC04MjU2LTFhODVlNmFiZGFjMyJ9LCJ0aXRsZSI6IkludGVybmFsIHBob3NwaG9ydXMgbG9hZGluZyBhbHRlcnMgbnV0cmllbnQgbGltaXRhdGlvbiBhbmQgY29udHJpYnV0ZXMgdG8gY3lhbm9iYWN0ZXJpYWwgYmxvb21zIGluIGEgcG9seW1pY3RpYyBsYWtlIiwiYWJzdHJhY3QiOiJBYnN0cmFjdCBDbGVhciBMYWtlLCBhIG1lZGl1bS1zaXplZCBoeXBlcmV1dHJvcGhpYywgcG9seW1pY3RpYyBsYWtlIGluIG5vcnRoZXJuIENhbGlmb3JuaWEsIGhhcyBoYWQgcmVjdXJyaW5nIGhhcm1mdWwgY3lhbm9iYWN0ZXJpYSBibG9vbXMgKEhDQnMpIGZvciBvdmVyIGEgY2VudHVyeSBkZXNwaXRlIHJlZHVjdGlvbnMgaW4gZXh0ZXJuYWwgcGhvc3Bob3J1cyAoUCkgbG9hZGluZ3MuIEludGVybmFsIFAgbG9hZGluZyBjYW4gYWx0ZXIgbnV0cmllbnQgYXZhaWxhYmlsaXR5IGFuZCBsaW1pdGF0aW9uIHN1cHBvcnRpbmcgSENCcyBidXQgaXMgcmFyZWx5IHF1YW50aWZpZWQgb3IgY29tcGFyZWQgd2l0aCBleHRlcm5hbCBsb2Fkcy4gV2UgaGF2ZSBxdWFudGlmaWVkIGV4dGVybmFsIFAgbG9hZHMgZnJvbSAyMDE5IHRvIDIwMjEgZm9yIHRoZSB0aHJlZSBtYWluIHRyaWJ1dGFyaWVzIChhY2NvdW50aW5nIGZvciA0NiUgb2YgdGhlIGZsb3cpIGFuZCBpbnRlcm5hbCBsb2FkaW5ncyB1c2luZyB0d28gbWV0aG9kczogYSBQIG1hc3MgYmFsYW5jZSBhbmQgbW9kZWxlZCByZWxlYXNlIHJhdGVzIG9mIHNvbHVibGUgcmVhY3RpdmUgcGhvc3Bob3J1cyBmcm9tIG94aWMgYW5kIGFub3hpYyBzZWRpbWVudHMuIEluIGFkZGl0aW9uLCB3ZSBjb21iaW5lZCBoaWdoLWZyZXF1ZW5jeSBpbiBzaXR1IG1lYXN1cmVtZW50cyBvZiB3YXRlciB0ZW1wZXJhdHVyZSBhbmQgZGlzc29sdmVkIG94eWdlbiwgZGlzY3JldGUgZ3JhYiBzYW1wbGluZyBmb3IgbnV0cmllbnQgY2hlbWlzdHJ5LCBhbmQgcmVtb3RlIHNlbnNpbmcgdG8gZXhwbG9yZSB0aGUgcG90ZW50aWFsIGRyaXZlcnMgb2YgdGhlIG9ic2VydmVkIHZhcmlhYmlsaXR5IGFuZCBwcm92aWRlIGEgY29tcHJlaGVuc2l2ZSB2aWV3IG9mIHRoZSBzcGF0aW90ZW1wb3JhbCBkeW5hbWljcyBvZiBIQ0JzLiBCeSB1bmRlcnN0YW5kaW5nIHRoZSByZWxhdGl2ZSBjb250cmlidXRpb24gb2YgZXh0ZXJuYWwgYW5kIGludGVybmFsIG51dHJpZW50IGxvYWRpbmdzIGFuZCB0aGUgcmVsYXRpb25zaGlwIGJldHdlZW4gZW52aXJvbm1lbnRhbCBwYXJhbWV0ZXJzIGFuZCBIQ0JzLCBpbnRlcmFubnVhbCBibG9vbSB2YXJpYWJpbGl0eSBjYW4gYmUgYmV0dGVyIHByZWRpY3RlZC4gQ29tcGFyYXRpdmUgZXN0aW1hdGVzIG9mIGV4dGVybmFsIGFuZCBpbnRlcm5hbCBwaG9zcGhvcnVzIGxvYWRpbmcgaW5kaWNhdGUgdGhhdCBpbnRlcm5hbCBzb3VyY2VzIGFjY291bnRlZCBmb3IgNzDigJM5NSUgb2YgdGhlIHRvdGFsIFAgaW5wdXQgaW50byB0aGUgc3lzdGVtIGR1cmluZyB0aGUgc3R1ZHkgcGVyaW9kLiBDb250cmFyeSB0byBvdGhlciBsYWtlcywgdGhlIGludGVuc2l0eSBvZiB0aGUgc3VtbWVyIGJsb29tIHNlYXNvbiB3YXMgY29ycmVsYXRlZCB0byB0aGUgdGltaW5nIGFuZCBkdXJhdGlvbiBvZiBhbm94aWEgcmF0aGVyIHRoYW4gdGhlIG1hZ25pdHVkZSBvZiBzcHJpbmcgcnVub2ZmLiBJbnRlcm5hbGx5IHJlbGVhc2VkIFAgc2hpZnRlZCB0aGUgc3lzdGVtIGZyb20gcGhvc3Bob3J1cyB0byBuaXRyb2dlbiBsaW1pdGF0aW9uIGR1cmluZyB0aGUgc3VtbWVyLCBwb3RlbnRpYWxseSBmYXZvcmluZyB0aGUgcHJvbGlmZXJhdGlvbiBvZiBuaXRyb2dlbi1maXhpbmcgY3lhbm9iYWN0ZXJpYS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAwNy9zMDAwMjctMDI0LTAxMDQ1LTIiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vbGluay5zcHJpbmdlci5jb20vY29udGVudC9wZGYvMTAuMTAwNy9zMDAwMjctMDI0LTAxMDQ1LTIucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjQtMDMtMTIiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTExIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJBcXVhdGljIFNjaWVuY2VzIiwiaXNzdWUiOjIsInZvbHVtZSI6ODYsImltcGFjdEZhY3RvciI6IjIuODQifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiU3ByaW5nZXIgU2NpZW5jZStCdXNpbmVzcyBNZWRpYSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NzUsImNpdGVkQnlDb3VudCI6MjF9fSwicXVvdGUiOiJDb250cmFyeSB0byBvdGhlciBsYWtlcywgdGhlIGludGVuc2l0eSBvZiB0aGUgc3VtbWVyIGJsb29tIHNlYXNvbiB3YXMgY29ycmVsYXRlZCB0byB0aGUgdGltaW5nIGFuZCBkdXJhdGlvbiBvZiBhbm94aWEgcmF0aGVyIHRoYW4gdGhlIG1hZ25pdHVkZSBvZiBzcHJpbmcgcnVub2ZmLiBJbnRlcm5hbGx5IHJlbGVhc2VkIFAgc2hpZnRlZCB0aGUgc3lzdGVtIGZyb20gcGhvc3Bob3J1cyB0byBuaXRyb2dlbiBsaW1pdGF0aW9uIGR1cmluZyB0aGUgc3VtbWVyLCBwb3RlbnRpYWxseSBmYXZvcmluZyB0aGUgcHJvbGlmZXJhdGlvbiBvZiBuaXRyb2dlbi1maXhpbmcgY3lhbm9iYWN0ZXJpYS4gLSBbQ2FsY2l1bSBhbmQgcGhvc3BoYXRlIG1ldGFib2xpYyBkaXNvcmRlcnNdKGh0dHBzOi8vbGluay5zcHJpbmdlci5jb20vc3ViamVjdHMvY2FsY2l1bS1hbmQtcGhvc3BoYXRlLW1ldGFib2xpYy1kaXNvcmRlcnMpIEV1dHJvcGhpY2F0aW9uIHBvc2VzIGEgZ3JhdmUgdGhyZWF0IHRvIGxlbnRpYyBlY29zeXN0ZW1zIGdsb2JhbGx5LCBwcm9tb3RpbmcgdGhlIHByb2xpZmVyYXRpb24gb2YgaGFybWZ1bCBjeWFub2JhY3RlcmlhIGJsb29tcyAoSENCcykgdGhhdCByZWR1Y2UgYXF1YXRpYyBiaW9kaXZlcnNpdHksIHlpZWxkaW5nIGluY3JlYXNlZCB3YXRlciB0cmVhdG1lbnQgY29zdHMsIGFuZCB0aHJlYXRlbmluZyBodW1hbiBoZWFsdGggdGhyb3VnaCBibG9vbS1wcm9kdWNlZCBjeWFub3RveGlucyAoQXLDoW96IGV0IGFsLiJ9XQ%3D%3D. Studies in polymictic lakes demonstrate that anoxia duration directly correlates with bloom intensity, shifting nutrient limitation from nitrogen to phosphorus and favoring N₂-fixing cyanobacteria (Nõges et al., 2020; Swann et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRpaW5hIiwiZmFtaWx5IjoiTsO1Z2VzIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hc2ltZSIsImZhbWlseSI6IkphbmF0aWFuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlJlZXQiLCJmYW1pbHkiOiJMYXVnYXN0ZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQZWV0ZXIiLCJmYW1pbHkiOiJOw7VnZXMifX1dLCJpZHMiOnsiaXNzbiI6WyIyMzUxLTk4OTQiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMTYvai5nZWNjby4yMDIwLmUwMTM2OSIsIm1hZyI6IjMxMDU0NDYwOTAiLCJvcGVuYWxleCI6IlczMTA1NDQ2MDkwIiwiZXhhIjoiaHR0cHM6Ly93d3cuc2NpZW5jZWRpcmVjdC5jb20vc2NpZW5jZS9hcnRpY2xlL3BpaS9TMjM1MTk4OTQyMDMwOTEwMCJ9LCJ0aXRsZSI6IlBvc3Qtc292aWV0IGNoYW5nZXMgaW4gbml0cm9nZW4gYW5kIHBob3NwaG9ydXMgc3RvaWNoaW9tZXRyeSBpbiB0d28gbGFyZ2Ugbm9uLXN0cmF0aWZpZWQgbGFrZXMgYW5kIHRoZSBpbXBhY3Qgb24gcGh5dG9wbGFua3RvbiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2ouZ2VjY28uMjAyMC5lMDEzNjkiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2ouZ2VjY28uMjAyMC5lMDEzNjkiLCJpc3N1ZWREYXRlIjoiMjAyMC0xMS0yMCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTEiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6Ikdsb2JhbCBFY29sb2d5IGFuZCBDb25zZXJ2YXRpb24iLCJ2b2x1bWUiOjI0LCJpbXBhY3RGYWN0b3IiOiIzLjU5In0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkVsc2V2aWVyIEJWIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo3MSwiY2l0ZWRCeUNvdW50IjoxNH19LCJxdW90ZSI6IlJhbmRvbSBGb3Jlc3QgYW5hbHlzaXMgaW5kaWNhdGVkIGFuIGltcG9ydGFudCByb2xlIG9mIGxpZ2h0IGxpbWl0YXRpb24gaW4gYm90aCBsYWtlcy4gQ29uc3RhbnRseSBsb3dlciBsZXZlbHMgb2YgTiBpbiB0aGUgZGVlcGVyIExha2UgUGVpcHNpIGZhdm91cmVkIE4yLWZpeGluZyBzcGVjaWVzLCB3aGljaCwgYXMgYSBwYXJhZG94LCBiZWNhbWUgUC1saW1pdGVkLiBBcyBjbGltYXRlIHdhcm1pbmcgcmVpbmZvcmNlcyBldXRyb3BoaWNhdGlvbiBwaGVub21lbmEgaW4gbGFrZXMgYnkgaW5jcmVhc2luZyBpbnRlcm5hbCBudXRyaWVudCBsb2FkaW5nIGFuZCBmYXZvdXJpbmcgYmxvb20tZm9ybWluZyBjeWFub2JhY3RlcmlhLCBtb3JlIHN0cmluZ2VudCBtZWFzdXJlcyB3b3VsZCBiZSBuZWVkZWQgdG8gZnVydGhlciBsaW1pdCBudXRyaWVudCBsb2FkcyAoZXNwZWNpYWxseSB0aGF0IG9mIFApIHRvIGxha2VzIHRocm91Z2ggaW1wcm92ZWQgd2FzdGV3YXRlciB0cmVhdG1lbnQgYW5kIGluY3JlYXNlZCBlZmZpY2llbmN5IG9mIGZlcnRpbGlzZXIgYXBwbGljYXRpb24uIExvbmctdGVybSBjaGFuZ2VzIGluIG51dHJpZW50IGxvYWRpbmcgYW5kIHN0b2ljaGlvbWV0cnkgaW4gd2F0ZXIgYm9kaWVzIHJlZmxlY3QgY2hhbmdlcyBpbiBhZ3JpY3VsdHVyYWwgcHJhY3RpY2VzIGFuZCB3YXN0ZXdhdGVyIHRyZWF0bWVudC4ifSx7Im9yaWdpbiI6NSwid29yayI6eyJ0eXBlIjo4LCJzdWJ0eXBlIjp7Im9uZW9mS2luZCI6ImFydGljbGVUeXBlIiwiYXJ0aWNsZVR5cGUiOjF9LCJjb250cmlidXRvcnMiOlt7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNaWNhaCBNLiIsImZhbWlseSI6IlN3YW5uIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFsaWNpYSIsImZhbWlseSI6IkNvcnTDqXMifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQWxleGFuZGVyIEwuIiwiZmFtaWx5IjoiRm9ycmVzdCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJOaWNob2xhcyBULiIsImZhbWlseSI6IkZyYW1zdGVkIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlN0ZXZlbiIsImZhbWlseSI6IlNhZHJvIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikdlb2ZmcmV5IiwiZmFtaWx5IjoiU2NobGFkb3cifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW5nZWxhIERlIiwiZmFtaWx5IjoiUGFsbWHigJBEb3cifX1dLCJpZHMiOnsiaXNzbiI6WyIxMDE1LTE2MjEiLCIxNDIwLTkwNTUiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMDcvczAwMDI3LTAyNC0wMTA0NS0yIiwib3BlbmFsZXgiOiJXNDM5MjcxMzM5OSIsImV4YSI6Imh0dHBzOi8vbGluay5zcHJpbmdlci5jb20vYXJ0aWNsZS8xMC4xMDA3L3MwMDAyNy0wMjQtMDEwNDUtMj9lcnJvcj1jb29raWVzX25vdF9zdXBwb3J0ZWQmY29kZT04M2NmOGQ4OC1jOWEwLTRjZGQtODI1Ni0xYTg1ZTZhYmRhYzMifSwidGl0bGUiOiJJbnRlcm5hbCBwaG9zcGhvcnVzIGxvYWRpbmcgYWx0ZXJzIG51dHJpZW50IGxpbWl0YXRpb24gYW5kIGNvbnRyaWJ1dGVzIHRvIGN5YW5vYmFjdGVyaWFsIGJsb29tcyBpbiBhIHBvbHltaWN0aWMgbGFrZSIsImFic3RyYWN0IjoiQWJzdHJhY3QgQ2xlYXIgTGFrZSwgYSBtZWRpdW0tc2l6ZWQgaHlwZXJldXRyb3BoaWMsIHBvbHltaWN0aWMgbGFrZSBpbiBub3J0aGVybiBDYWxpZm9ybmlhLCBoYXMgaGFkIHJlY3VycmluZyBoYXJtZnVsIGN5YW5vYmFjdGVyaWEgYmxvb21zIChIQ0JzKSBmb3Igb3ZlciBhIGNlbnR1cnkgZGVzcGl0ZSByZWR1Y3Rpb25zIGluIGV4dGVybmFsIHBob3NwaG9ydXMgKFApIGxvYWRpbmdzLiBJbnRlcm5hbCBQIGxvYWRpbmcgY2FuIGFsdGVyIG51dHJpZW50IGF2YWlsYWJpbGl0eSBhbmQgbGltaXRhdGlvbiBzdXBwb3J0aW5nIEhDQnMgYnV0IGlzIHJhcmVseSBxdWFudGlmaWVkIG9yIGNvbXBhcmVkIHdpdGggZXh0ZXJuYWwgbG9hZHMuIFdlIGhhdmUgcXVhbnRpZmllZCBleHRlcm5hbCBQIGxvYWRzIGZyb20gMjAxOSB0byAyMDIxIGZvciB0aGUgdGhyZWUgbWFpbiB0cmlidXRhcmllcyAoYWNjb3VudGluZyBmb3IgNDYlIG9mIHRoZSBmbG93KSBhbmQgaW50ZXJuYWwgbG9hZGluZ3MgdXNpbmcgdHdvIG1ldGhvZHM6IGEgUCBtYXNzIGJhbGFuY2UgYW5kIG1vZGVsZWQgcmVsZWFzZSByYXRlcyBvZiBzb2x1YmxlIHJlYWN0aXZlIHBob3NwaG9ydXMgZnJvbSBveGljIGFuZCBhbm94aWMgc2VkaW1lbnRzLiBJbiBhZGRpdGlvbiwgd2UgY29tYmluZWQgaGlnaC1mcmVxdWVuY3kgaW4gc2l0dSBtZWFzdXJlbWVudHMgb2Ygd2F0ZXIgdGVtcGVyYXR1cmUgYW5kIGRpc3NvbHZlZCBveHlnZW4sIGRpc2NyZXRlIGdyYWIgc2FtcGxpbmcgZm9yIG51dHJpZW50IGNoZW1pc3RyeSwgYW5kIHJlbW90ZSBzZW5zaW5nIHRvIGV4cGxvcmUgdGhlIHBvdGVudGlhbCBkcml2ZXJzIG9mIHRoZSBvYnNlcnZlZCB2YXJpYWJpbGl0eSBhbmQgcHJvdmlkZSBhIGNvbXByZWhlbnNpdmUgdmlldyBvZiB0aGUgc3BhdGlvdGVtcG9yYWwgZHluYW1pY3Mgb2YgSENCcy4gQnkgdW5kZXJzdGFuZGluZyB0aGUgcmVsYXRpdmUgY29udHJpYnV0aW9uIG9mIGV4dGVybmFsIGFuZCBpbnRlcm5hbCBudXRyaWVudCBsb2FkaW5ncyBhbmQgdGhlIHJlbGF0aW9uc2hpcCBiZXR3ZWVuIGVudmlyb25tZW50YWwgcGFyYW1ldGVycyBhbmQgSENCcywgaW50ZXJhbm51YWwgYmxvb20gdmFyaWFiaWxpdHkgY2FuIGJlIGJldHRlciBwcmVkaWN0ZWQuIENvbXBhcmF0aXZlIGVzdGltYXRlcyBvZiBleHRlcm5hbCBhbmQgaW50ZXJuYWwgcGhvc3Bob3J1cyBsb2FkaW5nIGluZGljYXRlIHRoYXQgaW50ZXJuYWwgc291cmNlcyBhY2NvdW50ZWQgZm9yIDcw4oCTOTUlIG9mIHRoZSB0b3RhbCBQIGlucHV0IGludG8gdGhlIHN5c3RlbSBkdXJpbmcgdGhlIHN0dWR5IHBlcmlvZC4gQ29udHJhcnkgdG8gb3RoZXIgbGFrZXMsIHRoZSBpbnRlbnNpdHkgb2YgdGhlIHN1bW1lciBibG9vbSBzZWFzb24gd2FzIGNvcnJlbGF0ZWQgdG8gdGhlIHRpbWluZyBhbmQgZHVyYXRpb24gb2YgYW5veGlhIHJhdGhlciB0aGFuIHRoZSBtYWduaXR1ZGUgb2Ygc3ByaW5nIHJ1bm9mZi4gSW50ZXJuYWxseSByZWxlYXNlZCBQIHNoaWZ0ZWQgdGhlIHN5c3RlbSBmcm9tIHBob3NwaG9ydXMgdG8gbml0cm9nZW4gbGltaXRhdGlvbiBkdXJpbmcgdGhlIHN1bW1lciwgcG90ZW50aWFsbHkgZmF2b3JpbmcgdGhlIHByb2xpZmVyYXRpb24gb2Ygbml0cm9nZW4tZml4aW5nIGN5YW5vYmFjdGVyaWEuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjEwMDcvczAwMDI3LTAyNC0wMTA0NS0yIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2xpbmsuc3ByaW5nZXIuY29tL2NvbnRlbnQvcGRmLzEwLjEwMDcvczAwMDI3LTAyNC0wMTA0NS0yLnBkZiIsImlzc3VlZERhdGUiOiIyMDI0LTAzLTEyIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMSIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiQXF1YXRpYyBTY2llbmNlcyIsImlzc3VlIjoyLCJ2b2x1bWUiOjg2LCJpbXBhY3RGYWN0b3IiOiIyLjg0In0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlNwcmluZ2VyIFNjaWVuY2UrQnVzaW5lc3MgTWVkaWEifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjc1LCJjaXRlZEJ5Q291bnQiOjIxfX0sInF1b3RlIjoiQ29udHJhcnkgdG8gb3RoZXIgbGFrZXMsIHRoZSBpbnRlbnNpdHkgb2YgdGhlIHN1bW1lciBibG9vbSBzZWFzb24gd2FzIGNvcnJlbGF0ZWQgdG8gdGhlIHRpbWluZyBhbmQgZHVyYXRpb24gb2YgYW5veGlhIHJhdGhlciB0aGFuIHRoZSBtYWduaXR1ZGUgb2Ygc3ByaW5nIHJ1bm9mZi4gSW50ZXJuYWxseSByZWxlYXNlZCBQIHNoaWZ0ZWQgdGhlIHN5c3RlbSBmcm9tIHBob3NwaG9ydXMgdG8gbml0cm9nZW4gbGltaXRhdGlvbiBkdXJpbmcgdGhlIHN1bW1lciwgcG90ZW50aWFsbHkgZmF2b3JpbmcgdGhlIHByb2xpZmVyYXRpb24gb2Ygbml0cm9nZW4tZml4aW5nIGN5YW5vYmFjdGVyaWEuIC0gW0NhbGNpdW0gYW5kIHBob3NwaGF0ZSBtZXRhYm9saWMgZGlzb3JkZXJzXShodHRwczovL2xpbmsuc3ByaW5nZXIuY29tL3N1YmplY3RzL2NhbGNpdW0tYW5kLXBob3NwaGF0ZS1tZXRhYm9saWMtZGlzb3JkZXJzKSBFdXRyb3BoaWNhdGlvbiBwb3NlcyBhIGdyYXZlIHRocmVhdCB0byBsZW50aWMgZWNvc3lzdGVtcyBnbG9iYWxseSwgcHJvbW90aW5nIHRoZSBwcm9saWZlcmF0aW9uIG9mIGhhcm1mdWwgY3lhbm9iYWN0ZXJpYSBibG9vbXMgKEhDQnMpIHRoYXQgcmVkdWNlIGFxdWF0aWMgYmlvZGl2ZXJzaXR5LCB5aWVsZGluZyBpbmNyZWFzZWQgd2F0ZXIgdHJlYXRtZW50IGNvc3RzLCBhbmQgdGhyZWF0ZW5pbmcgaHVtYW4gaGVhbHRoIHRocm91Z2ggYmxvb20tcHJvZHVjZWQgY3lhbm90b3hpbnMgKEFyw6FveiBldCBhbC4ifV0%3D. In Chilika Lagoon, post-monsoon blooms have induced widespread anoxia, with DO crashes blocking light penetration by up to 80% and amplifying scum persistence (Nair & Nayak, 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hdnlhIFZpa3JhbWFuIiwiZmFtaWx5IjoiTmFpciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQcmF0ZWVwIEt1bWFyIiwiZmFtaWx5IjoiTmF5YWsifX1dLCJpZHMiOnsiaXNzbiI6WyIyMjk2LTc3NDUiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzODkvZm1hcnMuMjAyMy4xMDg3Mjk2Iiwib3BlbmFsZXgiOiJXNDM4MjcyMzEzMyJ9LCJ0aXRsZSI6IlVuY292ZXJpbmcgd2F0ZXIgcXVhbGl0eSBhbmQgZXZhbHVhdGluZyB2dWxuZXJhYmlsaXRpZXMgb2Ygc21hbGwtc2NhbGUgZmlzaGVyaWVzIGluIENoaWxpa2EgTGFnb29uLCBJbmRpYSIsImFic3RyYWN0IjoiU21hbGwtc2NhbGUgZmlzaGVyaWVzIChTU0ZzKSBwbGF5IGEgc3Ryb25nIHJvbGUgaW4gc3VzdGFpbmluZyBtaWxsaW9ucyBvZiBsaXZlbGlob29kcywgZm9vZCBzZWN1cml0eSwgbnV0cml0aW9uLCBhbmQgaW5jb21lIGdsb2JhbGx5IGJ1dCB0aGUgZmlzaGVycyBlbmdhZ2VkIGluIHRoaXMgc2VjdG9yIHNpbXVsdGFuZW91c2x5IGV4cGVyaWVuY2UgaGlnaCBsZXZlbHMgb2YgdnVsbmVyYWJpbGl0eSBhbmQgcHJvY2Vzc2VzIG9mIG1hcmdpbmFsaXNhdGlvbi4gU2V2ZXJhbCBmYWN0b3JzIGFyZSBhdHRyaWJ1dGVkIHRvIHRoZSBtdWx0aWRpbWVuc2lvbmFsIHZ1bG5lcmFiaWxpdGllcyB0aGUgc21hbGwtc2NhbGUgZmlzaGVycyBleHBlcmllbmNlLCBzcGFubmluZyBib3RoIG5hdHVyYWwgKGUuZy4sIG5hdHVyYWwgZGlzYXN0ZXJzLCBlY29zeXN0ZW0gY2hhbmdlKSBhbmQgYW50aHJvcG9nZW5pYyAoZS5nLiwgcG9saWN5IGNoYW5nZSwgaHlkcm9sb2dpY2FsIGludGVydmVudGlvbnMsIGFxdWFjdWx0dXJlKSBwcmVzc3VyZXMuIFdoaWxlIHRoZXJlIGlzIG11Y2ggbGl0ZXJhdHVyZSBvbiB2YXJpb3VzIG5hdHVyYWwgYW5kIGh1bWFuIGRyaXZlcnMgb2YgdnVsbmVyYWJpbGl0eSBpbiBzbWFsbC1zY2FsZSBmaXNoZXJ5IGNvbW11bml0aWVzLCBhbiBhYnNlbmNlIG9mIHJlc2VhcmNoIGNvbm5lY3RpbmcgdnVsbmVyYWJpbGl0eSB3aXRoIHdhdGVyIHF1YWxpdHkgaXMgZXZpZGVudC4gRmlzaGVyIGNvbW11bml0aWVzIG9mdGVuIHRhbGsgYWJvdXQgZmlzaCBpbiByZWxhdGlvbiB0byB0aGUgaGVhbHRoIG9mIHRoZWlyIGFxdWF0aWMgaGFiaXRhdHMgd2hlcmVpbiB3YXRlciBxdWFsaXR5IGlzIHNlZW4gYXMgYSBrZXkgcGFyYW1ldGVyLiBUaGUgbGluayBiZXR3ZWVuIGhlYWx0aHkgZmlzaCBhbmQgZ29vZCBxdWFsaXR5IHdhdGVyIGhhcyBzaWduaWZpY2FudCBpbXBsaWNhdGlvbnMgZm9yIHN0cm9uZyBhbmQgdmlhYmxlIGZpc2hpbmcgY29tbXVuaXRpZXMuIFRoaXMgcGFwZXIgZXhhbWluZXMgdGhlc2UgbGlua3MgZnVydGhlciBieSBmb2N1c2luZyBvbiB0aGUgbmF0dXJlIG9mIHZ1bG5lcmFiaWxpdGllcyBjYXVzZWQgYnkgd2F0ZXIgcXVhbGl0eSBjaGFuZ2VzIGluIHRoZSBzbWFsbC1zY2FsZSBmaXNoZXJ5IHN5c3RlbSBvZiBDaGlsaWthIExhZ29vbiBpbiBJbmRpYS4gV2UgdW5kZXJ0YWtlIGRldGFpbGVkIGFuYWx5c2lzIG9mIHRoZSBpbnZhc2l2ZSBzaHJpbXAgYXF1YWN1bHR1cmUgYWN0aXZpdGllcyBhbmQgaHlkcm9sb2dpY2FsIGludGVydmVudGlvbnMgZm9yIG9wZW5pbmcgb2YgYSBsYWdvb24gaW5sZXQgd2l0aCB0aGUgQmF5IG9mIEJlbmdhbCBhcyB0d28gZG9taW5hbnQgZHJpdmVycyBhZHZlcnNlbHkgaW1wYWN0aW5nIHdhdGVyIHF1YWxpdHkgYW5kIGluY3JlYXNpbmcgdnVsbmVyYWJpbGl0aWVzIG9mIHRoZSBlbnRpcmUgc21hbGwtc2NhbGUgZmlzaGVyaWVzIHNvY2lhbC1lY29sb2dpY2FsIHN5c3RlbS4gT3VyIGFuYWx5c2lzIHN1Z2dlc3RzIHRoYXQgdGhlcmUgYXJlIHN0cm9uZyBpbnRlcmNvbm5lY3Rpb25zIGJldHdlZW4gY2hhbmdlcyBpbiB3YXRlciBxdWFsaXR5IGFuZCB0aGUgbGV2ZWxzIG9mIHZ1bG5lcmFiaWxpdGllcyBpbiB0aGUgU1NGcyBvZiBDaGlsaWthIExhZ29vbi4gUG9sbHV0YW50cyBzdWNoIGFzIHBlc3RpY2lkZXMsIGFuZCBvcmdhbmljIGNvbXBvdW5kcyBhY2N1bXVsYXRlIGluIGZpc2ggdGlzc3VlcyBhbmQgYWZmZWN0IHRoZWlyIGdyb3d0aCwgcmVwcm9kdWN0aW9uLCBhbmQgb3ZlcmFsbCBoZWFsdGguIFRoaXMgbGVkIHRvIGRlY2xpbmVzIGluIGZpc2ggcG9wdWxhdGlvbnMsIG1ha2luZyBpdCBtb3JlIGRpZmZpY3VsdCBmb3IgZmlzaGVycyB0byBtYWtlIGEgbGl2aW5nLiBJbiBhZGRpdGlvbiB0byBkaXJlY3QgaW1wYWN0cyBvbiBmaXNoIHBvcHVsYXRpb25zLCBwb29yIHdhdGVyIHF1YWxpdHkgYWxzbyBoYXMgaW5kaXJlY3QgZWZmZWN0cyBvbiB0aGUgc29jaWFsIGFuZCBlY29ub21pYyB2dWxuZXJhYmlsaXR5IG9mIFNTRnMuIEZvciBleGFtcGxlLCBjb250YW1pbmF0aW9uIG9mIHdhdGVyIHNvdXJjZXMgbGVkIHRvIHRoZSByZWR1Y2VkIG51bWJlciBvZiBmaXNoIHNwZWNpZXMgcmVkdWNpbmcgdGhlIGFtb3VudCBvZiB0aW1lIGZpc2hlcnMgY2FuIHNwZW5kIG9uIGZpc2hpbmcgYWN0aXZpdGllcy4gVGhpcyBhbHNvIGFmZmVjdGVkIHRoZSBtYXJrZXRhYmlsaXR5IG9mIGZpc2ggcHJvZHVjdHMsIHJlZHVjZWQgaW5jb21lIGFuZCBpbmNyZWFzZWQgcG92ZXJ0eS4gVG8gZnVsbHkgdW5kZXJzdGFuZCB0aGUgaW50ZXJjb25uZWN0aW9ucyBiZXR3ZWVuIHdhdGVyIHF1YWxpdHkgYW5kIHZ1bG5lcmFiaWxpdHkgaW4gU1NGcyBpbiBDaGlsaWthIExhZ29vbiwgaXQgaXMgaW1wb3J0YW50IHRvIGNvbnNpZGVyIGJvdGggZW52aXJvbm1lbnRhbCBhbmQgc29jaWFsIGZhY3RvcnMsIGFzIHdlbGwgYXMgdGhlIGNvbXBsZXggZmVlZGJhY2sgbG9vcHMgYmV0d2VlbiB0aGVzZSBmYWN0b3JzLiBUaGUgc3R1ZHkgaGVscHMgaW4gYnJpZGdpbmcgYSBjcnVjaWFsIGdhcCBpbiBvdXIgdW5kZXJzdGFuZGluZyBvZiB0aGUgcm9sZSBvZiB3YXRlciBxdWFsaXR5IGluIHZ1bG5lcmFiaWxpdHkgYW5hbHlzaXMgd2l0aGluIHJlc291cmNlIGRlcGVuZGVudCBjb21tdW5pdGllcy4gV2UgY29uY2x1ZGUgd2l0aCBrZXkgaW5zaWdodHMgb24gcG9zc2libGUgY29waW5nIHJlc3BvbnNlcyBhbmQgYWRhcHRpdmUgY2FwYWNpdHkgbmVjZXNzYXJ5IGZvciB0aGUgc21hbGwtc2NhbGUgZmlzaGVyaWVzIGNvbW11bml0aWVzIHRvIHRyYW5zaXRpb24gdG93YXJkIHZpYWJpbGl0eS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMzM4OS9mbWFycy4yMDIzLjEwODcyOTYiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vd3d3LmZyb250aWVyc2luLm9yZy9hcnRpY2xlcy8xMC4zMzg5L2ZtYXJzLjIwMjMuMTA4NzI5Ni9wZGYiLCJpc3N1ZWREYXRlIjoiMjAyMy0wNi0yOSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZyb250aWVycyBpbiBNYXJpbmUgU2NpZW5jZSIsInZvbHVtZSI6MTAsImltcGFjdEZhY3RvciI6IjQuMjgifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRnJvbnRpZXJzIE1lZGlhIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo1MSwiY2l0ZWRCeUNvdW50Ijo1fX0sInF1b3RlIjoiV2F0ZXIgcXVhbGl0eSBvZiBDaGlsaWthIExhZ29vbiBoYXMgZGVncmFkZWQgZHVlIHRvIGFudGhyb3BvZ2VuaWMgaW50ZXJ2ZW50aW9ucyBhbmQgbmF0dXJhbCBmb3JjZXMgb2YgY2hhbmdlLiBWYXJpb3VzIGFjdGl2aXRpZXMgaW4gdGhlIENoaWxpa2EgTGFnb29uLCBzdWNoIGFzIGluZHVzdHJpYWwgd2FzdGV3YXRlciBkaXNwb3NhbCwgc2V3YWdlIGR1bXBpbmcsIGFxdWFjdWx0dXJlLCBoeWRyb2xvZ2ljYWwgaW50ZXJ2ZW50aW9ucywgYW5kIGN5Y2xvbmVzLCBhcmUgcHJvZHVjaW5nIHNhbGluaXR5IGNoYW5nZXMsIHNlZGltZW50IGRlcG9zaXRpb24sIG51dHJpZW50IGVucmljaG1lbnQsIGV1dHJvcGhpY2F0aW9uLCBhbmQgZGVhZCB6b25lcywgYXMgc3RhdGVkLiBGaXNoIHN1cnZpdmFsIGlzIGF0IHJpc2sgYXMgYSByZXN1bHQgb2YgdGhlIHNpZ25pZmljYW50IHJlZHVjdGlvbiBpbiBlbnZpcm9ubWVudGFsIGNpcmN1bXN0YW5jZXMsIHdoaWNoIGRpbWluaXNoZXMgZmlzaGVybWVu4oCZcyBpbmNvbWUgYW5kIHB1c2hlcyB0aGVtIG91dCBvZiBzb2NpZXR5LiBUaGVzZSBjYXVzZXMgYWZmZWN0aW5nIHRoZSB3YXRlciBxdWFsaXR5IG9mIHRoZSBMYWdvb24gZGVwaWN0IG1hbnkgZmFjZXMgb2YgdnVsbmVyYWJpbGl0eSBpbiBTU0YgY29tbXVuaXRpZXMuIEFzIHRoZSBDaGlsaWthIExhZ29vbiBlbnZpcm9ubWVudCBpcyBleHBlY3RlZCB0byB1bmRlcmdvIGluY3JlYXNpbmdseSBkcmFzdGljIGNoYW5nZXMgKHN1Y2ggYXMgY2F0Y2ggY29tcG9zaXRpb24sIGNhdGNoIGNhcGFjaXR5LCBiaW9kaXZlcnNpdHkgdmFyaWF0aW9ucywgd2F0ZXIgcXVhbGl0eSwgYW5kIGZpc2hlcnkgcmV2ZW51ZXMpLCBpbnRlcm5hdGlvbmFsIHBvbGljeSBoYXMgYmVjb21lIG1vcmUgaW50ZXJlc3RlZCBpbiBhZGFwdGF0aW9uIG1lYXN1cmVzLiBHaXZlbiBob3cgZGl2ZXJzZSBTU0YgZWNvc3lzdGVtcyBhcmUgZXhwZWN0ZWQgdG8gYmUgYmV0d2VlbiBjb250aW5lbnRzLCB0aGVyZSBpcyBubyDigJxvbmUgc2l6ZSBmaXRzIGFsbOKAnSBhcHByb2FjaC4gVGhlIHBhcGVyIG91dGxpbmVkIHNldmVyYWwgY29waW5nIGFuZCBhZGFwdGF0aW9uIG1lY2hhbmlzbXMgdG8gaW1wcm92ZSB3YXRlciBxdWFsaXR5IGFuZCBjb250aW51ZSBmaXNoaW5nIGluIGEgc3VzdGFpbmFibGUgd2F5LiBBZGFwdGF0aW9uIHN0cmF0ZWdpZXMgY2FuIG9jY2FzaW9uYWxseSBoYXZlIHVuZm9yZXNlZW4gY29uc2VxdWVuY2VzIGZvciBmaXNoZXJpZXMgYW5kIGZpc2hpbmcgY29tbXVuaXRpZXMuIEFueSB1bmV4cGVjdGVkIGNvbnNlcXVlbmNlcyBvZiBhZGFwdGF0aW9uIGNhbiBiZSBtaXRpZ2F0ZWQgdGhyb3VnaCBsb25nLXRlcm0gcGxhbm5pbmcgYW5kIGlkZW50aWZpY2F0aW9uIG9mIGZ1dHVyZSBvcHRpb25zIHRocm91Z2ggc2NlbmFyaW8gYW5hbHlzaXMuIFRoZSBmaW5kaW5ncyBvZiB0aGlzIHBhcGVyIGNhbiBiZSB1dGlsaXplZCB0byBkZXZlbG9wIHBvbGljaWVzIHRvIHByb3RlY3Qgd2F0ZXIgcXVhbGl0eSBhbmQgU1NGcyBpbiBhIHZhcmlldHkgb2YgY29hc3RhbCBsYWdvb24gc2V0dGluZ3MuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjIyLjI4NTAzNjA4NzAzNjEzMywibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ5MzA1MzQzNjI3OTMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjIzLjc3MzUxOTUxNTk5MTIxLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDgzMzY0MTA1MjI0NjEsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjI1LjI1NTU4NDcxNjc5Njg3NSwibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ4OTU1OTE3MzU4Mzk4NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjYuNzQ0MDY4MTQ1NzUxOTUzLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNTA2ODA1NDE5OTIxODc1LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyOC4yMjYxMzMzNDY1NTc2MTcsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40OTA0NjMyNTY4MzU5NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjkuNzE0NjE2Nzc1NTEyNjk1LCJsZWZ0Ijo4LjMzMTU5MjU1OTgxNDQ1Mywid2lkdGgiOjQwLjQ5NDA5ODY2MzMzMDA4LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozMS4xOTY2ODM4ODM2NjY5OTIsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40OTQzOTIzOTUwMTk1MywiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MzIuNjg1MTY1NDA1MjczNDQsImxlZnQiOjguMzMxNTk2Mzc0NTExNzE5LCJ3aWR0aCI6NDAuNDg3NDQ1ODMxMjk4ODMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM0LjE2NzIzMjUxMzQyNzczNCwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40OTc3NDU1MTM5MTYwMTYsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM1LjY1NTcxNTk0MjM4MjgxLCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ5ODYzODE1MzA3NjE3LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozNy4xMzc3ODMwNTA1MzcxMSwibGVmdCI6MTAuNjg1MTg2Mzg2MTA4Mzk4LCJ3aWR0aCI6MzguMTQxMTA1NjUxODU1NDcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM4LjYyNjI2NjQ3OTQ5MjE5LCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4NzE1MjA5OTYwOTM3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDAuMTA4MzMzNTg3NjQ2NDg0LCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4OTg1NjcxOTk3MDcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQxLjU5NjgxMzIwMTkwNDMsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDg4NjY2NTM0NDIzODMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQzLjA3ODg4MDMxMDA1ODU5NCwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40ODE4NjExMTQ1MDE5NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDQuNTY3MzYzNzM5MDEzNjcsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDgzMDU4OTI5NDQzMzYsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQ2LjA0OTQzMDg0NzE2Nzk3LCJsZWZ0Ijo4LjMzMzEwODkwMTk3NzUzOSwid2lkdGgiOjQwLjUwMDc0NzY4MDY2NDA2LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo0Ny41Mzc5MTQyNzYxMjMwNSwibGVmdCI6OC4zMzMxMDg5MDE5Nzc1MzksIndpZHRoIjo0MC40OTU2MDU0Njg3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDkuMDE5OTgxMzg0Mjc3MzQ0LCJsZWZ0Ijo4LjMzMzEwNjA0MDk1NDU5LCJ3aWR0aCI6NDAuNTA2ODE2ODY0MDEzNjcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUwLjUwODQ2NDgxMzIzMjQyLCJsZWZ0Ijo4LjMzMzEwNjA0MDk1NDU5LCJ3aWR0aCI6NDAuNDg4NTA2MzE3MTM4NjcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUxLjk5MDUyODEwNjY4OTQ1LCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDk3NzI2NDQwNDI5NjksImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUzLjQ3OTAxMTUzNTY0NDUzLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6MzMuMTU5NDQ2NzE2MzA4NTk0LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo1NC45NjEwNzg2NDM3OTg4MywibGVmdCI6MTAuNjgzNjcwMDQzOTQ1MzEyLCJ3aWR0aCI6MzguMTM2NTk2Njc5Njg3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NTYuNDQ5NTYyMDcyNzUzOTA2LCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDg3MTQ0NDcwMjE0ODQ0LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEzIn19XQ%3D%3D. Such deoxygenation not only mobilizes legacy nutrients but also releases greenhouse gases like methane during anaerobic remineralization, reinforcing climate feedbacks (Meerhoff et al., 2022; Paerl et al., 2019)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhhbnMgVy4iLCJmYW1pbHkiOiJQYWVybCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLYXJsIEUuIiwiZmFtaWx5IjoiSGF2ZW5zIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hdGhhbiBTLiIsImZhbWlseSI6IkhhbGwifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiVGltb3RoeSBHLiIsImZhbWlseSI6Ik90dGVuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1lbmd5dWFuIiwiZmFtaWx5IjoiWmh1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhhaSIsImZhbWlseSI6Ilh1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikd1YW5nd2VpIiwiZmFtaWx5IjoiWmh1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkJvcWlhbmciLCJmYW1pbHkiOiJRaW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIxMzIzLTE2NTAiLCIxNDQ4LTYwNTkiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwNzEvbWYxODM5MiIsIm1hZyI6IjI5NDIzMjE1NDIiLCJvcGVuYWxleCI6IlcyOTQyMzIxNTQyIn0sInRpdGxlIjoiTWl0aWdhdGluZyBhIGdsb2JhbCBleHBhbnNpb24gb2YgdG94aWMgY3lhbm9iYWN0ZXJpYWwgYmxvb21zOiBjb25mb3VuZGluZyBlZmZlY3RzIGFuZCBjaGFsbGVuZ2VzIHBvc2VkIGJ5IGNsaW1hdGUgY2hhbmdlIiwiYWJzdHJhY3QiOiJNYW5hZ2luZyBhbmQgbWl0aWdhdGluZyB0aGUgZ2xvYmFsIGV4cGFuc2lvbiBvZiB0b3hpYyBjeWFub2JhY3RlcmlhbCBoYXJtZnVsIGFsZ2FsIGJsb29tcyAoQ3lhbm9IQUJzKSBpcyBhIG1ham9yIGNoYWxsZW5nZSBmYWNpbmcgcmVzZWFyY2hlcnMgYW5kIHdhdGVyIHJlc291cmNlIG1hbmFnZXJzLiBWYXJpb3VzIGFwcHJvYWNoZXMsIGluY2x1ZGluZyBudXRyaWVudCBsb2FkIHJlZHVjdGlvbiwgYXJ0aWZpY2lhbCBtaXhpbmcgYW5kIGZsdXNoaW5nLCBvbW5pdm9yb3VzIGZpc2ggcmVtb3ZhbCwgYWxnYWVjaWRlIGFwcGxpY2F0aW9ucyBhbmQgc2VkaW1lbnQgZHJlZGdpbmcsIGhhdmUgYmVlbiB1c2VkIHRvIHJlZHVjZSBibG9vbSBvY2N1cnJlbmNlcy4gSG93ZXZlciwgbWFuYWdlcnMgbm93IGZhY2UgdGhlIGFkZGl0aW9uYWwgY2hhbGxlbmdlIG9mIGhhdmluZyB0byBhZGRyZXNzIHRoZSBlZmZlY3RzIG9mIGNsaW1hdGUgY2hhbmdlIG9uIHdhdGVyc2hlZCBoeWRyb2xvZ2ljYWwgYW5kIG51dHJpZW50IGxvYWQgZHluYW1pY3MsIHdhdGVyIHRlbXBlcmF0dXJlLCBtaXhpbmcgcmVnaW1lIGFuZCBpbnRlcm5hbCBudXRyaWVudCBjeWNsaW5nLiBSaXNpbmcgdGVtcGVyYXR1cmVzIGFuZCBpbmNyZWFzaW5nIGZyZXF1ZW5jaWVzIGFuZCBtYWduaXR1ZGVzIG9mIGV4dHJlbWUgd2VhdGhlciBldmVudHMsIGluY2x1ZGluZyB0cm9waWNhbCBjeWNsb25lcywgZXh0cmF0cm9waWNhbCBzdG9ybXMsIGZsb29kcyBhbmQgZHJvdWdodHMsIGFsbCBwcm9tb3RlIEN5YW5vSEFCcyBhbmQgYWZmZWN0IHRoZSBlZmZpY2FjeSBvZiBlY29zeXN0ZW0gcmVtZWRpYXRpb24gbWVhc3VyZXMuIFRoZXNlIGNsaW1hdGljIGNoYW5nZXMgd2lsbCBsaWtlbHkgcmVxdWlyZSBzZXR0aW5nIHN0cmljdGVyIG51dHJpZW50IChpbmNsdWRpbmcgYm90aCBuaXRyb2dlbiBhbmQgcGhvc3Bob3J1cykgcmVkdWN0aW9uIHRhcmdldHMgZm9yIGJsb29tIGNvbnRyb2wgaW4gYWZmZWN0ZWQgd2F0ZXJzLiBJbiBhZGRpdGlvbiwgdGhlIGVmZmljYWN5IG9mIGN1cnJlbnRseSB1c2VkIG1ldGhvZHMgdG8gcmVkdWNlIEN5YW5vSEFCcyB3aWxsIG5lZWQgdG8gYmUgcmUtZXZhbHVhdGVkIGluIGxpZ2h0IG9mIHRoZSBzeW5lcmdpc3RpYyBlZmZlY3RzIG9mIGNsaW1hdGUgY2hhbmdlIHdpdGggbnV0cmllbnQgZW5yaWNobWVudC4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTA3MS9tZjE4MzkyIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2Nkci5saWIudW5jLmVkdS9kb3dubG9hZHMvMDU3NDIyODM0IiwiaXNzdWVkRGF0ZSI6IjIwMTktMDMtMjUiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTEwIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJNYXJpbmUgYW5kIEZyZXNod2F0ZXIgUmVzZWFyY2giLCJpc3N1ZSI6NSwidm9sdW1lIjo3MSwiaW1wYWN0RmFjdG9yIjoiMi43MCJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjU3OSwibGFzdFBhZ2UiOjU3OX19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJDU0lSTyBQdWJsaXNoaW5nIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjoxMDQsImNpdGVkQnlDb3VudCI6OTF9fSwicXVvdGUiOiJUaGF0IHJlbGF0aW9uc2hpcCBoYXMgc2VydmVkIGFzIHRoZSBmb3VuZGF0aW9uIGZvciBzZXR0aW5nICBudXRyaWVudCBsb2FkaW5nIHRhcmdldHMgaW4gbGFrZXMgYXJvdW5kIHRoZSB3b3JsZCwgYW5kIGNhcmVmdWxseSAgZGVzaWduZWQgZXhwZXJpbWVudHMgbWF5IGVsdWNpZGF0ZSB0byB3aGF0IGV4dGVudCB0ZW1wZXJhdHVyZSAgaW5jcmVhc2UgYWZmZWN0cyB0aGUgdGhyZXNob2xkIGF0IHdoaWNoIHRoZSBmcmVxdWVuY3kgb2Ygb2NjdXIgIHJlbmNlIG9mIEN5YW5vSEFCcyBxdWlja2x5IHJpc2VzIHdpdGggaW5jcmVhc2luZyBOIGFuZCBQLiBDb25jbHVzaW9ucyBSZWNlbnQgc3R1ZGllcyBpbmRpY2F0ZSB0aGF0IHRoZSBzeW5lcmdpc3RpYyBjb21iaW5hdGlvbiBvZiBhbnRocm9wb2dlbmljIG51dHJpZW50IGxvYWRpbmcsIHJpc2luZyB0ZW1wZXJhdHVyZXMsIGVuaGFuY2VkICB2ZXJ0aWNhbCBzdHJhdGlmaWNhdGlvbiwgaW5jcmVhc2VkIHJlc2lkZW5jZSB0aW1lIGFuZCBtb3JlICBleHRyZW1lIGNsaW1hdGljIGNvbmRpdGlvbnMgb3ZlcmFsbCB3aWxsIGZhdm91ciBjeWFub2JhY3RlcmlhbCAgZG9taW5hbmNlIGluIGEgd2lkZSByYW5nZSBvZiBhcXVhdGljIGVjb3N5c3RlbXMuIFRoZSBleHBhbiAgc2lvbiBvZiBDeWFub0hBQnMgaGFzIHNlcmlvdXMgY29uc2VxdWVuY2VzIGZvciBodW1hbiB3YXRlciAgc3VwcGxpZXMsIGZpc2hlcmllcyBhbmQgcmVjcmVhdGlvbmFsIHJlc291cmNlcy4gRWNvc3lzdGVtIHNjYWxlICBwaHlzaWNhbCBjaGVtaWNhbCBiaW90aWMgbWl0aWdhdGlvbiBzdHJhdGVnaWVzIHdpbGwgaGF2ZSB0byAgaW5jb3Jwb3JhdGUgbnV0cmllbnQgKG1vc3Qgb2Z0ZW4gYm90aCBOIGFuZCBQKSBpbnB1dCByZWR1Y3Rpb25zLiAgVGhlIG1hZ25pdHVkZXMgb2YgdGhlc2UgcmVkdWN0aW9ucyB3aWxsIG5lZWQgdG8gYmUgc3lzdGVtICBzcGVjaWZpYyBhbmQgd2lsbCBsaWtlbHkgbmVlZCB0byBiZSBhZGp1c3RlZCB3aXRoIGNoYW5naW5nIGNsaSAgbWF0aWMgY29uZGl0aW9ucy4gU3BlY2lmaWNhbGx5LCB3YXRlciBxdWFsaXR5IG1hbmFnZXJzIHdpbGwgaGF2ZSAgdG8gYWNjb21tb2RhdGUgd2FybWluZyBhbmQgZ3JlYXRlciBoeWRyb2xvZ2ljYWwgdmFyaWFiaWxpdHkgIGFuZCBleHRyZW1lbmVzcyByZXN1bHRpbmcgZnJvbSBjbGltYXRlIGNoYW5nZSBpbiBmb3JtdWxhdGluZyAgdGhlaXIgc3RyYXRlZ2llcy4gQSBrZXkgbG9uZyB0ZXJtIGNvbnRyb2wgd2UgY2FuIGV4ZXJ0IHRvIHJlZHVjZSAgdGhlIHJhdGUgYW5kIGV4dGVudCBvZiBnbG9iYWwgd2FybWluZyBpcyBjdXJiaW5nIGdyZWVuaG91c2UgZ2FzICBlbWlzc2lvbnMuIFdpdGhvdXQgdGhpcyBlc3NlbnRpYWwgc3RlcCwgZnV0dXJlIHdhcm1pbmcgdHJlbmRzLCAgaHlkcm9sb2dpY2FsIGV4dHJlbWVzIChyZWNvcmQgcmFpbmZhbGxzIGFuZCBkcm91Z2h0cykgYW5kIHRoZWlyICBlZmZlY3RzIG9uIGFxdWF0aWMgZWNvc3lzdGVtcyB3aWxsIGNvbnRpbnVlIHRvIHByb21vdGUgb3Bwb3IgIHR1bmlzdGljIEN5YW5vSEFCcy4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6MTQuMTQ5MjMzODE4MDU0MiwibGVmdCI6MTMuMTI4NTk1MzUyMTcyODUyLCJ3aWR0aCI6MzcuMDgyMTcyMzkzNzk4ODMsImhlaWdodCI6MS41MTY2MDIyNzc3NTU3MzczfX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjE1LjQ1MTUwODUyMjAzMzY5MSwibGVmdCI6MTMuMTI4NTQ0ODA3NDM0MDgyLCJ3aWR0aCI6MzcuMDg0ODMxMjM3NzkyOTcsImhlaWdodCI6MS41MTY2MDMyMzE0MzAwNTM3fX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjE2Ljc1Mzc4NDE3OTY4NzUsImxlZnQiOjEzLjEyODU0MjkwMDA4NTQ1LCJ3aWR0aCI6MzcuMDgzMjA5OTkxNDU1MDgsImhlaWdodCI6MS41MTY2MDQxODUxMDQzNzAxfX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjE4LjA1NjA1MzE2MTYyMTA5NCwibGVmdCI6MTMuMTI4NTQ0ODA3NDM0MDgyLCJ3aWR0aCI6MzYuNjE1MDc0MTU3NzE0ODQ0LCJoZWlnaHQiOjEuNTE2NjA0MTg1MTA0MzcwMX19LHsicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjoxOS40NjgzNzgwNjcwMTY2LCJsZWZ0IjoxMy4xMjg1NDQ4MDc0MzQwODIsIndpZHRoIjozNC4yOTA1MzExNTg0NDcyNjYsImhlaWdodCI6MS4yNDUwMTEyMTA0NDE1ODk0fX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjIyLjIyNjE0Mjg4MzMwMDc4LCJsZWZ0IjoxMy4xMjg1NTA1Mjk0Nzk5OCwid2lkdGgiOjcuNjQ5ODczNzMzNTIwNTA4LCJoZWlnaHQiOjEuMTM1NDgzMTQ1NzEzODA2Mn19LHsicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjoyMy43MzQ4NDk5Mjk4MDk1NywibGVmdCI6MTMuMTI4NTM1MjcwNjkwOTE4LCJ3aWR0aCI6MzcuMDg5Mjg2ODA0MTk5MjIsImhlaWdodCI6MjYuMDg4NjQwMjEzMDEyNjk1fX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNTg5In19LHsib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik0uIiwiZmFtaWx5IjoiTWVlcmhvZmYifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSi4iLCJmYW1pbHkiOiJBdWRldCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJULkEuIiwiZmFtaWx5IjoiRGF2aWRzb24ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTC4gRGUiLCJmYW1pbHkiOiJNZWVzdGVyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlMuIiwiZmFtaWx5IjoiSGlsdCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTLiIsImZhbWlseSI6Iktvc3RlbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJaYWlodWEiLCJmYW1pbHkiOiJMaXUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTi4iLCJmYW1pbHkiOiJNYXp6ZW8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSGFucyBXLiIsImZhbWlseSI6IlBhZXJsIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik0uIiwiZmFtaWx5IjoiU2NoZWZmZXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiRS4iLCJmYW1pbHkiOiJKZXBwZXNlbiJ9fV0sImlkcyI6eyJpc3NuIjpbXSwiaXNibiI6W10sImRvaSI6IjEwLjE3NjE1Lzd2ZTUtdzAxNiIsIm9wZW5hbGV4IjoiVzQzMTg0MTU1NzcifSwidGl0bGUiOiJGZWVkYmFjayBiZXR3ZWVuIGNsaW1hdGUgY2hhbmdlIGFuZCBldXRyb3BoaWNhdGlvbjogcmV2aXNpdGluZyB0aGUgYWxsaWVkIGF0dGFjayBjb25jZXB0IGFuZCBob3cgdG8gc3RyaWtlIGJhY2siLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTc2MTUvN3ZlNS13MDE2IiwicmVzb3VyY2VVcmwiOiJodHRwczovL2Nkci5saWIudW5jLmVkdS9kb3dubG9hZHMvaDk4OXJjOTM4IiwiaXNzdWVkRGF0ZSI6IjIwMjItMDEtMDEiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTEwIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJDYXJvbGluYSBEaWdpdGFsIFJlcG9zaXRvcnkgKFVuaXZlcnNpdHkgb2YgTm9ydGggQ2Fyb2xpbmEgYXQgQ2hhcGVsIEhpbGwpIiwiaW1wYWN0RmFjdG9yIjoiMC4xMCJ9LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJVbml2ZXJzaXR5IG9mIE5vcnRoIENhcm9saW5hIGF0IENoYXBlbCBIaWxsIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjowLCJjaXRlZEJ5Q291bnQiOjN9fSwicXVvdGUiOiJXaXRoIGNsaW1hdGUgd2FybWluZywgdGhlIGRpcmVjdCBhbmQgaW5kaS1yZWN0IGVmZmVjdHMgb2YgaW5jcmVhc2VkIGxha2UgdGVtcGVyYXR1cmVzIHdpbGwgbGlrZWx5IGRyaXZlIGNoYW5nZXMgaW4gY3lhbm9iYWN0ZXJpYWwgdGF4YSBhbmQgdGhlaXIgcHJvZHVjLXRpb24gb2YgY3lhbm9iYWN0ZXJpYWwgdG94aW5zLCBsaWtlbHkgaW5jbHVkaW5nIHByb21vdC1pbmcgdGhlIHNlbGVjdGlvbiBvZiBhIGZldyBNaWNyb2N5c3RpcyBzcGVjaWVzIG9yIHN0cmFpbnMgcHJvZHVjaW5nIGhpZ2hseSB0b3hpYyBtaWNyb2N5c3Rpbi1MUiAoTWFudHpvdWtpIGV0IGFsLiAyMDE4KS4gRGVzcGl0ZSB2YXJpYXRpb25zIGFtb25nIGxha2VzIGluIGEgd2F0ZXItc2NhcGUsIGEgcmVnaW9uYWwgaW5jcmVhc2UgaW4gbWljcm9jeXN0aW4gbWF4aW1hIGFuZCBkdXJhdGlvbiBvZiBibG9vbXMgaGFzIGFsc28gYmVlbiBjbGVhcmx5IGV4cGxhaW5lZCBieSBpbmNyZWFzaW5nIHRlbXBlcmF0dXJlIGluIGNlbnRyYWwgTm9ydGggQW1lcmljYSAoSGF5ZXMgZXQgYWwuIDIwMjApLCB3aXRoIGluY3JlYXNpbmcgcmlza3MgdG8gaHVtYW4gaGVhbHRoLiBHcm93aW5nIGV2aWRlbmNlIGluZGljYXRlcyB0aGF0IGN5YW5vYmFjdGVyaWFsIGRvbWluYW5jZSB3aWxsIGV4ZXJ0IGEgcG9zaXRpdmUgZmVlZGJhY2sgd2l0aCBjbGltYXRlIHdhcm1pbmcgKEhhdmVucyBldCBhbC4gMjAxNiwgUGFlcmwgZXQgYWwuIDIwMjApLiBNYW55IGN5YW5vYmFjdGVyaWFsIHNwZWNpZXMgcHJvbGlmZXJhdGUgb24gdGhlIGxha2Ugc3VyZmFjZSwgYWJzb3JiaW5nIGxpZ2h0LCBlbmhhbmNpbmcgbG9jYWxpemVkIHRlbXBlcmF0dXJlIGluY3JlYXNlcywgYW5kIGluY3JlYXNpbmcgdGhlcm1hbCBzdHJhdGlmaWNhdGlvbiBwb3RlbnRpYWwgKEthaHJ1IGV0IGFsLiAxOTkzKS4gQ3lhbm9iYWN0ZXJpYSBwcm9tb3RlIHRoZSByZWxlYXNlIG9mIEdIR3MsIHBhcnRpY3VsYXJseSBDSDQgKFlhbiBldCBhbC4gMjAxNyksIGR1cmluZyB0aGUgZGVjb21wb3NpdGlvbiBmb2xsb3dpbmcgdGhlIGNvbGxhcHNlIG9mIGEgYmxvb20uIEN5YW5vYmFjdGVyaWEgYWxzbyBwcm9tb3RlIHJlbWluZXJhbGl6YXRpb24gb2YgbnV0cmktZW50cyBkdXJpbmcgYmxvb21zLCBsZWFkaW5nIHRvIG11bHRpcGxlIGJsb29tcyBpbiBhIHNpbmdsZSBzZWFzb24uIFVuZGVyIGV1dHJvcGhpYyBhbmQgd2FybWluZyBjb25kaS10aW9ucywgY3lhbm9iYWN0ZXJpYSBkb21pbmFuY2UgbWF5IHRodXMgYmUgYSBzZWxmLXBlcnBldHVhdGluZyBwcm9waGVjeSAoRmlnLiAyIGFuZCAzKS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo2Ny4wNjM0MDAyNjg1NTQ2OSwibGVmdCI6NTEuNDc4ODAxNzI3Mjk0OTIsIndpZHRoIjozOC42NzM0MjM3NjcwODk4NDQsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6NjguNjk4NTQ3MzYzMjgxMjUsImxlZnQiOjUxLjQ3ODgwMTcyNzI5NDkyLCJ3aWR0aCI6MzguNjc5NDk2NzY1MTM2NzIsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6NzAuMzMzNjg2ODI4NjEzMjgsImxlZnQiOjUxLjQ3ODgwMTcyNzI5NDkyLCJ3aWR0aCI6MzguNjc3NTgxNzg3MTA5Mzc1LCJoZWlnaHQiOjEuNDcxMTAxNTIyNDQ1Njc4N319LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjcxLjk2ODgyNjI5Mzk0NTMxLCJsZWZ0Ijo1MS40Nzg4MDE3MjcyOTQ5Miwid2lkdGgiOjM4LjY3OTEwMzg1MTMxODM2LCJoZWlnaHQiOjEuNDcxMTAxNTIyNDQ1Njc4N319LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjczLjYxMDU3MjgxNDk0MTQsImxlZnQiOjUxLjQ3ODgwMTcyNzI5NDkyLCJ3aWR0aCI6MzguNjc0NDI3MDMyNDcwNywiaGVpZ2h0IjoxLjQ3Mzc0Njg5NTc5MDF9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo3NS4yNDcwMzk3OTQ5MjE4OCwibGVmdCI6NTEuNDc4ODA5MzU2Njg5NDUsIndpZHRoIjozOC42NzQyNjMwMDA0ODgyOCwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo3Ni44ODIxODY4ODk2NDg0NCwibGVmdCI6NTEuNDc4ODA5MzU2Njg5NDUsIndpZHRoIjozOC42ODExNzE0MTcyMzYzMywiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo3OC41MjUyNTMyOTU4OTg0NCwibGVmdCI6NTEuNDc4ODA5MzU2Njg5NDUsIndpZHRoIjozOC42NzU2NjI5OTQzODQ3NjYsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODAuMTYwNDAwMzkwNjI1LCJsZWZ0Ijo1MS40Nzg4MDkzNTY2ODk0NSwid2lkdGgiOjM4LjY4MjAyOTcyNDEyMTA5NCwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo4MS43OTU1Mzk4NTU5NTcwMywibGVmdCI6NTEuNDc4ODA5MzU2Njg5NDUsIndpZHRoIjozOC42NzQ5ODM5NzgyNzE0ODQsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODMuNDM4NjEzODkxNjAxNTYsImxlZnQiOjUxLjQ3ODgwOTM1NjY4OTQ1LCJ3aWR0aCI6MzguNjc2MTg1NjA3OTEwMTU2LCJoZWlnaHQiOjEuNDg2OTczNTI0MDkzNjI4fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODUuMDg5NjMwMTI2OTUzMTIsImxlZnQiOjUxLjQ3ODgwOTM1NjY4OTQ1LCJ3aWR0aCI6NC42ODI4MDUwNjEzNDAzMzIsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODYuNzI0Nzc3MjIxNjc5NjksImxlZnQiOjUzLjQ0MTE3NzM2ODE2NDA2LCJ3aWR0aCI6Ni4xNDAzNTU1ODcwMDU2MTUsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODYuNzI0ODkxNjYyNTk3NjYsImxlZnQiOjYwLjk5NDg3Njg2MTU3MjI2Niwid2lkdGgiOjYuMDM2OTkyNTQ5ODk2MjQsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODYuNzI0ODkxNjYyNTk3NjYsImxlZnQiOjY4LjQ0NjUyNTU3MzczMDQ3LCJ3aWR0aCI6Ni4xNTA3MDU4MTQzNjE1NzIsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODYuNzI0ODkxNjYyNTk3NjYsImxlZnQiOjc2LjAwOTgyNjY2MDE1NjI1LCJ3aWR0aCI6Mi43MjczMzMzMDcyNjYyMzU0LCJoZWlnaHQiOjEuNDcxMTAxNTIyNDQ1Njc4N319LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjg2LjcyNDg5MTY2MjU5NzY2LCJsZWZ0Ijo4MC4xNDk1ODE5MDkxNzk2OSwid2lkdGgiOjEwLjAwODIzNDk3NzcyMjE2OCwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo4OC4zNjAwMzg3NTczMjQyMiwibGVmdCI6NTEuNDc3NjU3MzE4MTE1MjM0LCJ3aWR0aCI6MzguNjcyMDU0MjkwNzcxNDg0LCJoZWlnaHQiOjEuNDcxMTAxNTIyNDQ1Njc4N319LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjkwLjAwMzExMjc5Mjk2ODc1LCJsZWZ0Ijo1MS40Nzc2NTczMTgxMTUyMzQsIndpZHRoIjozOC42NzY0OTg0MTMwODU5NCwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo5MS42MzgyNTIyNTgzMDA3OCwibGVmdCI6NTEuNDc3NjUzNTAzNDE3OTcsIndpZHRoIjozOC42NzU4MzQ2NTU3NjE3MiwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjksImJib3giOnsidG9wIjo2LjQ2ODkzNTk2NjQ5MTY5OSwibGVmdCI6OS44NDAxMzM2NjY5OTIxODgsIndpZHRoIjozOC42ODYzMjUwNzMyNDIxOSwiaGVpZ2h0IjoxNi4yMTIxMjU3NzgxOTgyNDJ9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI5In19XQ%3D%3D.

Aquatic "dead zones" expanses of near-anoxic water uninhabitable to most metazoans emerge as blooms collapse, collapsing food webs from the bottom up. Benthic macroinvertebrates, vital for nutrient cycling and as fish prey, suffocate first, followed by sediment-dependent fish species (Haxthausen, 2021)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkthcmwgUmFzbXVzIEF1Z3VzdCIsImZhbWlseSI6IkhheHRoYXVzZW4ifX1dLCJpZHMiOnsiaXNzbiI6W10sImlzYm4iOltdLCJtYWciOiIzMTc3MTEzMjA3Iiwib3BlbmFsZXgiOiJXMzE3NzExMzIwNyJ9LCJ0aXRsZSI6IlNlZGltZW50IG1pY3JvYmlhbCBmdWVsIGNlbGxzIGZvciBsYWtlIHJlc3RvcmF0aW9uIiwiYWJzdHJhY3QiOiJFdXRyb3BoaWNhdGlvbiBvZiBsYWtlcyBpcyB1YmlxdWl0b3VzLCBhcyAzMC00MCAlIG9mIHRoZSB3b3JsZOKAmXMgbGFrZXMgYXJlIGV1dHJvcGhpYywgYW5kIGNvbmRpdGlvbnMgY29udGludW91c2x5IGRldGVyaW9yYXRpbmcuIEV1dHJvcGhpY2F0aW9uIGlzIGEgY29uZGl0aW9uIG9mIG92ZXJmZXJ0aWxpemF0aW9uLCB3aGVyZSBhbGwgcmVxdWlyZW1lbnRzIGZvciByYXBpZCBncm93dGggb2YgYWxnYWUgYXJlIG1ldCwgcmVzdWx0aW5nIGluIGFsZ2FsIGJsb29tcy4gVGhpcyBjYXVzZXMgYSBzZWxmLXN1c3RhaW5lZCwgdmljaW91cyBjeWNsZSB3aGVyZSBwb29yIGVjb2xvZ2ljYWwgY29uZGl0aW9ucyBhcmUgbWFpbnRhaW5lZCB1bnRpbCBudXRyaWVudHMgYXJlIGxvc3QgZnJvbSB0aGUgbGFrZSBvdmVyIG1hbnkgeWVhcnMuIExha2UgcmVzdG9yYXRpb24gc2Vla3MgdG8gYnJlYWsgdGhpcyB2aWNpb3VzIGN5Y2xlLiBJbmFkZXF1YXRlIGF2YWlsYWJpbGl0eSBvZiBhbnkgc2luZ2xlIG51dHJpZW50IGluIHdpbGwgbGltaXQgYWxnYWwgZ3Jvd3RoLCB0eXBpY2FsbHkgcGhvc3Bob3J1cyBpcyB0YXJnZXRlZC4gVGhlIHNjb3BlIG9mIHRoaXMgUGhEIHRoZXNpcyB3YXMgdG8gZGV2ZWxvcCwgdGVzdCBhbmQgb3B0aW1pemUgc2VkaW1lbnQgbWljcm9iaWFsIGZ1ZWwgY2VsbHMgKFNNRkMpIGFzIGEgbm92ZWwgbWV0aG9kIGZvciBsYWtlIHJlc3RvcmF0aW9uLiBUaGUgdGhlc2lzIHNwYW5zIHRoZSB0d28gbWFpbiB0b3BpY3Mgb2YgU01GQ3MgYW5kIGxha2UgcmVzdG9yYXRpb24gdGVjaG5pcXVlcy4gTGFrZSByZXN0b3JhdGlvbiB0ZWNobmlxdWVzIGFyZSByZXZpZXdlZCwgcHJvdmlkaW5nIHVuZGVyc3RhbmRpbmcgb2YgdGhlIHVuZGVybHlpbmcgbWVjaGFuaXNtcy4gU01GQ3MgYXJlIGFwcHJvYWNoZWQgZnJvbSBhbiBlbmdpbmVlcmluZyBwZXJzcGVjdGl2ZSBhbmQgYSBmb2N1cyBvbiBsYWtlIHJlbWVkaWF0aW9uLiBUaGUgc3BlY2lmaWMgb2JqZWN0aXZlcyB3ZXJlIHRvIHRlc3QgYSBsb3ctY29zdCBlbGVjdHJvZGUgbWF0ZXJpYWwgZm9yIGl0cyBkdXJhYmlsaXR5LCBwcmFjdGljYWxpdHksIGFuZCBwZXJmb3JtYW5jZS4gUGVyZm9ybWFuY2Ugd2FzIGFzc2Vzc2VkIGFzIHRoZSBhYmlsaXR5IHRvIGxpbWl0IHBob3NwaG9ydXMgcmVsZWFzZSBmcm9tIHNlZGltZW50cywgaW5oaWJpdCBtZXRoYW5lIHJlbGVhc2UsIGFuZCB0aGUgYWRkZWQgYmVuZWZpdHMgb2YgZGVncmFkaW5nIHBvbGx1dGFudHMsIGFuZCBnZW5lcmF0aW5nIGVsZWN0cmljaXR5LiBQcm9vZiBvZiBjb25jZXB0IHdhcyBzaG93biBpbiBsYWJvcmF0b3J5IHJlYWN0b3JzICgxIEwpLCB0aGUgZWZmZWN0IG9mIHVwc2NhbGluZyB0byAyMDAgTCBpbiBuYXR1cmFsIGVudmlyb25tZW50IHdhcyBzdHVkaWVkIGluIG91dGRvb3IgbWVzb2Nvc21zLCBhbmQgdGhlbiB0ZXN0ZWQgYXQgcGlsb3Qgc2NhbGUgaW4gYSBldXRyb3BoaWMgZnJlc2h3YXRlciBsYWtlLiBQaWxvdCBpbnN0YWxsYXRpb25zIHdlcmUgdGhlIGxhcmdlc3QgaW4gdGhlIHdvcmxkIG9mIHRoZWlyIGtpbmQgYW5kIHdlcmUgb3BlcmF0ZWQgZm9yIDIgeWVhcnMuIFN0YWlubGVzcyBzdGVlbCB3YXMgc2VsZWN0ZWQgZm9yIHRlc3RpbmcgYXMgZWxlY3Ryb2RlIG1hdGVyaWFsIGR1ZSB0byBpdHMgbG93IGNvc3QsIGR1cmFiaWxpdHksIGFuZCBhdmFpbGFiaWxpdHkuIFBhaXJzIG9mIGVsZWN0cm9kZXMgd2VyZSBlbWJlZGRlZCBpbiB0aGUgc2VkaW1lbnQgKHRoaXMgaXMgY2FsbGVkIHRoZSBhbm9kZSkgYW5kIHN1c3BlbmRlZCBpbiB0aGUgb3Zlcmx5aW5nIHdhdGVyICh0aGlzIGlzIGNhbGxlZCB0aGUgY2F0aG9kZSkuIFRoZSB0d28gZWxlY3Ryb2RlcyB3ZXJlIGNvbm5lY3RlZCBhY3Jvc3MgYW4gZXh0ZXJuYWwgcmVzaXN0b3Igd2l0aCBlbGVjdHJpY2FsIHdpcmVzLiBNaWNyb2JlcyBpbiB0aGUgc2VkaW1lbnQgdXNlIHRoZSBhbm9kZSBwYWlyIGFzIGVsZWN0cm9uIGFjY2VwdG9yLCBlbmFibGluZyByZXNwaXJhdGlvbiBpbiB0aGUgYW5veGljIHNlZGltZW50LiBFbGVjdHJvbnMgdHJhdmVsIGZyb20gdGhlIHNlZGltZW50IHZpYSB0aGUgZWxlY3RyaWNhbCBjb25uZWN0b3IgdG8gdGhlIG92ZXJseWluZyBjYXRob2RlIHdoZXJlIG94eWdlbiBpbiB0aGUgd2F0ZXIgaXMgdGhlIHRlcm1pbmFsIGVsZWN0cm9uIGFjY2VwdG9yIGNvbXBsZXRpbmcgdGhlIGNpcmN1aXQuIEFzIGFuIGFkZGVkIGJlbmVmaXQsIGRlZ3JhZGF0aW9uIG9mIHBvbGx1dGFudHMgdXNpbmcgU01GQyB3YXMgdGVzdGVkIGluIG1hcmluZSBoYXJib3VyIHNlZGltZW50cywgYnV0IG5vIGNvbmNsdXNpdmUgcmVzdWx0cyBvYnRhaW5lZC4gTGFib3JhdG9yeSByZWFjdG9ycyB3ZXJlIG9wZXJhdGVkIGZvciBtb3JlIHRoYW4gMjAwIGRheXMgdXNpbmcgcGV0cm9sZXVtIGFzIHNvbGUgY2FyYm9uIHNvdXJjZSwgY29uZmlybWluZyBvaWwgZGVncmFkYXRpb24gYnkgU01GQ3MuIEEgcmVtb3ZhbCByYXRlIG9mIDAuOSBnIHBldHJvbGV1bSBtLTMgZC0xIHJlYWN0b3Igdm9sdW1lIHdhcyBlc3RpbWF0ZWQgZnJvbSBtZWFzdXJlZCBlbGVjdHJpY2FsIG91dHB1dC4gVGhpcyB3YXMgZG9uZSBieSBhc3N1bXB0aW9uIG9mIHRoZSBjaGVtaWNhbCByZWFjdGlvbiBvY2N1cnJpbmcsIGZyb20gdGhpcyB0aGUgbnVtYmVyIG9mIGVsZWN0cm9ucyB0cmFuc2ZlcnJlZCBmcm9tIG9pbCB0byB3YXRlciB3YXMga25vd24gYW5kIGNvdWxkIGJlIGNvcnJlbGF0ZWQgdG8gdGhlIG1lYXN1cmVkIGZsb3cgb2YgZWxlY3Ryb25zIChjdXJyZW50KSB0aHJvdWdoIHRoZSBleHRlcm5hbCBjaXJjdWl0LiBTdGFpbmxlc3Mgc3RlZWwgZWxlY3Ryb2RlcyB3ZXJlIG9wZXJhdGVkIGluIGZyZXNod2F0ZXIgbGFrZSBlbnZpcm9ubWVudHMgZm9yIG1vcmUgdGhhbiA0IHllYXJzIHdpdGggbWluaW1hbCBkYW1hZ2UgdG8gdGhlIG1hdGVyaWFsLiBUaGUgZWxlY3Ryb2RlIGlzIHRoZSBtYWluIG1hdGVyaWFsIGNvc3QgaW4gc3VjaCBzeXN0ZW1zLiBIb3dldmVyLCBBSVNJMzE2IHN0YWlubGVzcyBzdGVlbCBzaW1pbGFyIHRvIHdoYXQgd2FzIHVzZWQgaW4gdGhpcyB0aGVzaXMgd2FzIGF0IHRoZSB0aW1lIG9mIHdyaXRpbmcgYXZhaWxhYmxlIGF0IGJ1bGsgY29zdCBvZiAwLjA5MSDigqwgbS0yLCBjb3JyZXNwb25kaW5nIHRvIDkxMCDigqwgYWNyZS0xLCBzaWduaWZpY2FudGx5IGxvd2VyIHRoYW4gb3RoZXIgY29tbW9uIG1hdGVyaWFscy4gQ2F0aG9kZXMgYXQgbGFyZ2Ugc2NhbGUgY291bGQgYmUgbW92ZWQgaG9yaXpvbnRhbGx5IGFuZCB2ZXJ0aWNhbGx5IHdpdGggbWlub3IgaW1wYWN0IHRvIHBlcmZvcm1hbmNlLiBQaWxvdCBzY2FsZSBTTUZDcyBnZW5lcmF0ZWQgMC4zODMgbVcgbS0yIHN1c3RhaW5lZCBhbmQgMjcuNCBtVyBtLTIgcGVhayBvdXRwdXQuIFRoZSBzeXN0ZW0gd2FzIGxpbWl0ZWQgYnkgdGhlIGNhdGhvZGUgc3VyZmFjZSBhcmVhLCBtZWFuaW5nIG11bHRpLWxheWVyIGNhdGhvZGVzIHNob3VsZCBiZSB0ZXN0ZWQgaW4gdGhlIGZ1dHVyZSBpZiBlbGVjdHJpY2FsIHBvd2VyIHByb2R1Y3Rpb24gaXMgdGhlIGdvYWwuIFRoZSBjYXRob2RlIHNob3VsZCBiZSBwbGFjZWQgYXQgYSBkZXB0aCB0aGF0IGVuc3VyZXMgaGlnaCBkaXNzb2x2ZWQgb3h5Z2VuIGNvbmNlbnRyYXRpb25zIHRvIGF2b2lkIHRoZSBzeXN0ZW0gdG8gYmUgbGltaXRlZC4gSG93ZXZlciwgdG9vIGhpZ2ggZWxlY3RyaWNpdHkgZ2VuZXJhdGlvbiB3YXMgZm91bmQgY291bnRlcnByb2R1Y3RpdmUgZm9yIHJlbWVkaWF0aW9uIHB1cnBvc2VzIGJlY2F1c2UgdGhpcyB3b3VsZCBpbmNyZWFzZSBzZWRpbWVudCB0dXJub3ZlciByYXRlLCByZWxlYXNpbmcgcG9sbHV0YW50cyBzdG9yZWQgaW4gdGhlIHNlZGltZW50cy4gUCBiaW5kaW5nIHdhcyBub3QgZm91bmQgdG8gcmVxdWlyZSBoaWdoIHR1cm5vdmVyIG9mIHNlZGltZW50IG9yZ2FuaWMgbWF0dGVyLiBUaGUgdHVybm92ZXIgY2FuIGJlIGNvbnRyb2xsZWQgYnkgY2hhbmdpbmcgdGhlIGV4dGVybmFsIHJlc2lzdGFuY2UuIEhpZ2ggcmVzaXN0YW5jZSBsaW1pdHMgdGhlIGVsZWN0cmljYWwgcGVyZm9ybWFuY2UsIGxvdyByZXNpc3RhbmNlIGluY3JlYXNlcyBzZWRpbWVudCB0dXJub3ZlciBhbmQgcmlza3MgZGVzdGFiaWxpemluZyB0aGUgc3lzdGVtLiBUbyBhY2hpZXZlIHJlZHVjZWQgY29uY2VudHJhdGlvbnMgb2YgcGhvc3BoYXRlIGluIHRoZSB3YXRlciBjb2x1bW4sIGFub2RlcyBtdXN0IGJlIHBsYWNlZCBpbiB0aGUgc2VkaW1lbnQgc3VyZmFjZSBzaW5jZSBhIGZldyBtaWxsaW1ldHJlcyBvZiBzZWRpbWVudCBtYXkgY29udGFpbiBhbmQgcmVsZWFzZSBlbm91Z2ggUCB0byBzdXN0YWluIGV1dHJvcGhpY2F0aW9uLiBQaG9zcGhhdGUgd2FzIG1lYXN1cmVkIGluIHRoZSB3YXRlciBjb2x1bW4gbmVhciB0aGUgc2VkaW1lbnQsIGluIHN5c3RlbXMgd2l0aCBhbmQgd2l0aG91dCBlbGVjdHJvZGVzLiBQaG9zcGhhdGUgY29uY2VudHJhdGlvbnMgd2VyZSBzdWNjZXNzZnVsbHkgcmVkdWNlZCBieSB1cCB0byA5NSAlIGluIDItY2hhbWJlciBsYWJvcmF0b3J5IHJlYWN0b3JzLCBhbmQgYnkgdXAgdG8gOTggJSBpbiB0aGUgcGlsb3QgaW5zdGFsbGF0aW9uLiBJbiB0aGUgcGlsb3QgZXhwZXJpbWVudCwgcGhvc3BoYXRlIGNvbmNlbnRyYXRpb25zIGFib3ZlIGVsZWN0cm9kZXMgd2VyZSBsb3dlciBpbiBhbGwgbWVhc3VyZW1lbnRzLCBjb21wYXJlZCB0byBjb250cm9scy4gU2VkaW1lbnQgcmVkb3ggcG90ZW50aWFscyBpbmNyZWFzZWQgZnJvbSAtNDAwIG1WIHZzIHN0YW5kYXJkIGh5ZHJvZ2VuIGVsZWN0cm9kZSAoU0hFKSB0byAtMTc1IG1WIHZzIFNIRSwgY29ycmVzcG9uZGluZyB0byBhIG1vdmUgZnJvbSBhIG1ldGhhbm9nZW5ldGljIGRvbWFpbiB0byBzdWxwaHVyIHJlZHVjaW5nLiBBbmFseXNpcyBvZiB0aGUgbWljcm9iaWFsIGNvbW11bml0eSBpbmRpY2F0ZWQgZW5oYW5jZWQgc3VscGh1ciBjeWNsaW5nIHdoZXJlIGVsZWN0cm9kZXMgb3hpZGl6ZSBzdWxwaGlkZSB3aGljaCB0aGUgYmFjdGVyaWEgYWdhaW4gcmVkdWNlIHRvIG9idGFpbiBlbmVyZ3kuIEl0IHdhcyBoeXBvdGhlc2lzZWQgdGhhdCBTTUZDIHByb21vdGUgaGlnaGVyIHJlZG94IHBvdGVudGlhbHMgYnkgZmFjaWxpdGF0aW5nIG94aWRhdGlvbiBvZiBzdWxwaHVyIGFuZCBpcm9uIHNwZWNpZXMsIGluaGliaXRpbmcgSDJTIGZvcm1hdGlvbiBhbmQgRmUgc2NhdmVuZ2luZyBieSBGZVMgZm9ybWF0aW9uLCB0aHVzIGFsbG93aW5nIGZvciBQIGJpbmRpbmcuIEEgbG93ZXIgcmVsYXRpdmUgYWJ1bmRhbmNlIG9mIG1ldGhhbm9nZW5zIG9uIGVsZWN0cm9kZXMgY29tcGFyZWQgdG8gY29udHJvbHMgc3VnZ2VzdHMgc3VwcHJlc3Npb24gb3Igb3V0Y29tcGV0aW5nIG9mIG1ldGhhbm9nZW5zLlF1YW50aWZpY2F0aW9uIG9mIG1ldGhhbmUgcHJvZHVjdGlvbiB3YXMgZG9uZSBvbiB0aGUgYmFzaXMgb2YgYSBDSDQ6Q08yIHJhdGlvIHRvIGFsbG93IGNvbXBhcmFiaWxpdHkgYmV0d2VlbiBzeXN0ZW1zLiBNZXRoYW5lIGVtaXNzaW9uIHdhcyByZWR1Y2VkIGJ5IHVwIHRvIDk5ICUgYW5kIG1haW50YWluZWQgaW5oaWJpdGlvbiBmb3Igc2V2ZXJhbCBtb250aHMgaW4gbGFib3JhdG9yeSBzeXN0ZW1zIHdpdGggZWxlY3Ryb2Rlcy4gSW4gdGhlIHBpbG90IGluc3RhbGxhdGlvbnMgbm8gZ2FzIHNhbXBsZXMgY291bGQgYmUgY29sbGVjdGVkLCBhbmQgbm8gQ08yOkNINCByYXRpb3Mgd2VyZSBvYnRhaW5lZC4gQWVyYXRpb24gaXMgdGhlIGN1cnJlbnQgcmVzdG9yYXRpb24gdGVjaG5pcXVlIHRoYXQgbW9zdCBjbG9zZWx5IHJlc2VtYmxlcyB0aGUgcHJvcG9zZWQgU01GQyB0ZWNobmlxdWUuIEFlcmF0aW9uIGhhcyBwcmV2aW91c2x5IGZhaWxlZCB0byByZWR1Y2UgUCByZWxlYXNlIGluIHNldmVyYWwgc2VwYXJhdGUgaW52ZXN0aWdhdGlvbnMuIFNNRkNzIGFzIGEgcmVtZWRpYXRpb24gdG9vbCB3YXMgZm91bmQgdG8gb2ZmZXIgc2V2ZXJhbCBiZW5lZml0cyBvdmVyIGNvbnZlbnRpb25hbCBtZXRob2RzOiBTTUZDcyBvZmZlcnMgY29udGludW91cyBlZmZlY3QsIHdoaWxlIG90aGVyIHRlY2huaXF1ZXMgcmVxdWlyZSBjb250aW51b3VzIG9yIHJlcGVhdGVkIHRyZWF0bWVudC4gU01GQ3MgY2FuIGJlIGNvbnRyb2xsZWQgaW4gcmVhbC10aW1lIGJ5IGFsdGVyaW5nIHRoZSBleHRlcm5hbCByZXNpc3RvciBhbmQgY2FuIGJlIGVhc2lseSBtb25pdG9yZWQgYnkgbWVhc3VyaW5nIHRoZSBwcm9kdWNlZCBlbGVjdHJpY2FsIG91dHB1dC4gSWYgaW1wbGVtZW50ZWQgYXQgbGFyZ2Ugc2NhbGUgaXQgaXMgZXhwZWN0ZWQgdGhhdCB0aGUgZWxlY3Ryb2RlIGZpZWxkcyB3aWxsIGhlbHAgbWFpbnRhaW4gbG93IHBob3NwaGF0ZSBjb25jZW50cmF0aW9ucyBpbiB0aGUgd2F0ZXIgZm9yIGRlY2FkZXMgd2l0aCBtaW5pbWFsIGludGVydmVudGlvbiwgYW5kIGFmdGVyIHNvbWUgdGltZSwgYmVudGhpYyBwbGFudHMgYXJlIGV4cGVjdGVkIHRvIHJldHVybiwgdGh1cyBtYWludGFpbmluZyB0aGUgYmFsYW5jZSBoZW5jZSBmb3J0aC4iLCJ1cmwiOiJodHRwczovL2xvY2FsLmZvcnNrbmluZ3Nwb3J0YWwuZGsvbG9jYWwvZGtpLWNnaS93cy9jcmlzLWxpbms%2Fc3JjPWR0dSZpZD1kdHUtZDY2ZDMzNzAtMjU2Zi00ODg1LWEwOWUtMzM2YTkyNGY0YmVkJnRpPVNlZGltZW50JTIwbWljcm9iaWFsJTIwZnVlbCUyMGNlbGxzJTIwZm9yJTIwbGFrZSUyMHJlc3RvcmF0aW9uIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2JhY2tlbmQub3JiaXQuZHR1LmRrL3dzL2ZpbGVzLzI0ODA2OTU4OS9UaGVzaXNfb25saW5lX3ZlcnNpb25fS2FybF9IYXh0aGF1c2VuLnBkZiIsImlzc3VlZERhdGUiOiIyMDIxLTAxLTAxIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0wOCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiUmVzZWFyY2ggUG9ydGFsIERlbm1hcmsiLCJpbXBhY3RGYWN0b3IiOiIwLjA0In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6NzZ9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiVGVjaG5pY2FsIFVuaXZlcnNpdHkgb2YgRGVubWFyayJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MCwiY2l0ZWRCeUNvdW50IjoxfX0sInF1b3RlIjoiRXh0ZXJuYWwgaW5wdXRzIGluY3JlYXNlIFAgY29uY2VudHJhdGlvbiBpbiAgdGhlIHdhdGVyLCBjYXVzaW5nIGFsZ2FsIGJsb29tcyB0aGF0IHNoYWRvdyB0aGUgbGFrZSBib3R0b20sIGFuZCBiZW50aGljIG1hYy1yb3BoeXRlcyByZWNlZGUuIFByZWRhdG9yeSBmaXNoIGFyZSB1bmFibGUgdG8gaHVudCwgYW5kIHRoZWlyIGZvb2Qgc291cmNlIGlzICBkaXNydXB0ZWQgc3VjaCBhcyBmaXNoIGZlZWRpbmcgb24gYmVudGhpYyBpbnZlcnRlYnJhdGVzIGFuZCBib3R0b20tZHdlbGxpbmcgYW5pbWFscyBhcmUgc3VmZm9jYXRlZCAoVmFuZGVyIFphbmRlbiBhbmQgVmFkZWJvbmNvZXVyIDIwMDIpLCBzbyB0aGF0IHBsYW5rdGl2b3JvdXMgZmlzaCBzcGVjaWVzIGRvbWluYXRlLiBUaGlzIGluY3JlYXNlcyBwcmVkYXRpb24gb24gem9vIHBsYW5rdG9uICBhbmQgcmVkdWNlcyBncmF6aW5nIG9uIGFsZ2FlLiBBbGdhZSBwcm9saWZlcmF0ZSwgZGllLCBhbmQgc2luayB0byB0aGUgYm90dG9tLCAgY29uc3VtaW5nIG94eWdlbiB3aGVuIGRlZ3JhZGVkLCB3aGljaCBsZWFkcyB0byBveHlnZW4gZGVmaWNpdCBpbiB0aGUgYm90LXRvbSwga2lsbGluZyBmaXNoIChXdXJ0c2JhdWdoIGV0IGFsLiAyMDE5KS4gRmlndXJlIDI6IFZpY2lvdXMgY3ljbGVzIG9mIGV1dHJvcGhpY2F0aW9uLiBMZWZ0OiBJbnRlcmxpbmtlZCBwcm9jZXNzZXMgc3VzdGFpbmluZyBldXRyb3BoaS1jYXRpb24gaW4gYSBsYWtlLiBCbHVlOiBwcm9jZXNzZXMgaW4gd2F0ZXIsIGdyZWVuOiBwcm9jZXNzZXMgaW52b2x2aW5nIG1pY3JvYWxnYWUsIGdyZXk6IHNlZC1pbWVudCByZWxhdGVkIHByb2Nlc3NlcywgeWVsbG93OiBwcm9jZXNzZXMgaW52b2x2aW5nIGZhdW5hLiBSaWdodCwgZGFzaGVkIGxpbmU6IFRoZSBvcmRlciBpbiB3aGljaCBlbGVjdHJvbiBhY2NlcHRvcnMgYXJlIGNvbnN1bWVkIGluIHRoZSBhYnNlbmNlIG9mIE8yIHNob3duIGFzIGNvbmNlbnRyYXRpb24gdmVyc3VzICBkZXB0aCBpbiBzZWRpbWVudCwgYW5kIGF0IHdoaWNoIHBvdGVudGlhbCB0aGlzIG9jY3VycyAoQmVybmVyIDE5ODEsIGJ5IGNvdXJ0ZXN5IG9mIHRoZSBzb2NpZXR5ICBmb3Igc2VkaW1lbnRhcnkgZ2VvbG9neSkuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxNywiYmJveCI6eyJ0b3AiOjc5LjkzMzE1MTI0NTExNzE5LCJsZWZ0IjoxNC4yOTU1MDM2MTYzMzMwMDgsIndpZHRoIjo3MS45Mzg5MzQzMjYxNzE4OCwiaGVpZ2h0IjoyLjA1MzQ3NzI4NzI5MjQ4MDV9fSx7InBhZ2VOdW1iZXIiOjE3LCJiYm94Ijp7InRvcCI6ODEuOTcxMzQzOTk0MTQwNjIsImxlZnQiOjE0LjI5NTUwMzYxNjMzMzAwOCwid2lkdGgiOjcxLjMzMzAwMDE4MzEwNTQ3LCJoZWlnaHQiOjIuMDUzNDc3Mjg3MjkyNDgwNX19LHsicGFnZU51bWJlciI6MTcsImJib3giOnsidG9wIjo4NC4wMjM3OTYwODE1NDI5NywibGVmdCI6MTQuMjk1NTAzNjE2MzMzMDA4LCJ3aWR0aCI6NzEuODEyMDE5MzQ4MTQ0NTMsImhlaWdodCI6Mi4wNTM0NzcyODcyOTI0ODA1fX0seyJwYWdlTnVtYmVyIjoxNywiYmJveCI6eyJ0b3AiOjg2LjA2MTk5NjQ1OTk2MDk0LCJsZWZ0IjoxNC4yOTU1MDM2MTYzMzMwMDgsIndpZHRoIjo3MS45NTczNTkzMTM5NjQ4NCwiaGVpZ2h0IjoyLjA1MzQ3NzI4NzI5MjQ4MDV9fSx7InBhZ2VOdW1iZXIiOjE4LCJiYm94Ijp7InRvcCI6OS45MjY0ODc5MjI2Njg0NTcsImxlZnQiOjE0LjI5NTUwMzYxNjMzMzAwOCwid2lkdGgiOjcxLjk3NDA2MDA1ODU5Mzc1LCJoZWlnaHQiOjEwLjIzNDc3NzQ1MDU2MTUyM319LHsicGFnZU51bWJlciI6MTgsImJib3giOnsidG9wIjo0NC43MzQwNDMxMjEzMzc4OSwibGVmdCI6MTQuMjk1NTAzNjE2MzMzMDA4LCJ3aWR0aCI6NzEuODkzNDMyNjE3MTg3NSwiaGVpZ2h0IjoxMC40MDMyODk3OTQ5MjE4NzV9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI5MyJ9fV0%3D. Planktivores dominate temporarily but succumb to hypoxia-induced stress, disrupting trophic cascades and reducing zooplankton grazing pressure, which prolongs algal dominance (Haq et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjQsInN1YnR5cGUiOnt9LCJjb250cmlidXRvcnMiOlt7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJadWxmcWFydWwiLCJmYW1pbHkiOiJIYXEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU25vYmVyIiwiZmFtaWx5IjoiSXJzaGFkIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkF6bWF0IEFsYW0iLCJmYW1pbHkiOiJLaGFuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNoZWlraCBGLiIsImZhbWlseSI6IkFobWFkIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNob3drZWVuIE11emFtaWwiLCJmYW1pbHkiOiJCYXNoaXIifX1dLCJpZHMiOnsiaXNzbiI6WyIyNjMyLTA1MTciXSwiaXNibiI6W10sImRvaSI6IjEwLjU3NzIvaW50ZWNob3Blbi4xMDA2NTc4Iiwib3BlbmFsZXgiOiJXNDQwNDUyOTIwMyJ9LCJ0aXRsZSI6IkFkdmFuY2VzIGluIE1hbmFnaW5nIE5pdHJvZ2VuIGFuZCBQaG9zcGhvcnVzIEVtaXNzaW9ucyBpbiBSdW1pbmFudHM6IEEgSG9saXN0aWMgQXBwcm9hY2giLCJhYnN0cmFjdCI6IuKAnEJhbGFuY2luZyBOdXRyaXRpb24gYW5kIEVudmlyb25tZW50YWwgU3VzdGFpbmFiaWxpdHk6IEFkdmFuY2VzIGluIE1hbmFnaW5nIE5pdHJvZ2VuIGFuZCBQaG9zcGhvcnVzIEVtaXNzaW9ucyBpbiBSdW1pbmFudHPigJ0gZXhwbG9yZXMgdGhlIGludHJpY2F0ZSByZWxhdGlvbnNoaXAgYmV0d2VlbiBydW1pbmFudCBudXRyaXRpb24gYW5kIGVudmlyb25tZW50YWwgc3Rld2FyZHNoaXAuIEl0IGRlbHZlcyBpbnRvIHRoZSBkeW5hbWljcyBvZiBuaXRyb2dlbiBhbmQgcGhvc3Bob3J1cyBtZXRhYm9saXNtIGluIHJ1bWluYW50cywgYWRkcmVzc2luZyB0aGUgZW52aXJvbm1lbnRhbCByZXBlcmN1c3Npb25zIG9mIHRoZWlyIGVtaXNzaW9ucy4gVGhlc2UgZW1pc3Npb25zLCBwcmltYXJpbHkgaW4gdGhlIGZvcm0gb2YgYW1tb25pYSBhbmQgcGhvc3BoYXRlcywgcG9zZSBzaWduaWZpY2FudCBlbnZpcm9ubWVudGFsIGNoYWxsZW5nZXMsIGNvbnRyaWJ1dGluZyB0byBhaXIgYW5kIHdhdGVyIHBvbGx1dGlvbiwgc29pbCBkZWdyYWRhdGlvbiwgYW5kIGNsaW1hdGUgY2hhbmdlLiBUaGUgY2hhcHRlciBoaWdobGlnaHRzIGlubm92YXRpdmUgc3RyYXRlZ2llcyBhbmQgdGVjaG5vbG9naWNhbCBicmVha3Rocm91Z2hzIGFpbWVkIGF0IG1pdGlnYXRpbmcgbml0cm9nZW4gYW5kIHBob3NwaG9ydXMgZXhjcmV0aW9uLCBjcnVjaWFsIGZvciByZWR1Y2luZyBlbnZpcm9ubWVudGFsIHBvbGx1dGlvbi4gSXQgb2ZmZXJzIGluc2lnaHRzIGludG8gZGlldGFyeSBhZGp1c3RtZW50cywgZmVlZCBhZGRpdGl2ZXMsIGFuZCBiZXN0IHByYWN0aWNlcyBmb3Igc3VzdGFpbmFibGUgcnVtaW5hbnQgcHJvZHVjdGlvbi4gS2V5IGFyZWFzIG9mIGZvY3VzIGluY2x1ZGUgZGlldGFyeSBhZGp1c3RtZW50cyB0YWlsb3JlZCB0byBvcHRpbWl6ZSBudXRyaWVudCB1dGlsaXphdGlvbiBhbmQgcmVkdWNlIHdhc3RlLCB0aGUgdXRpbGl6YXRpb24gb2YgZmVlZCBhZGRpdGl2ZXMgdG8gZW5oYW5jZSBudXRyaWVudCBhYnNvcnB0aW9uIGFuZCBtaW5pbWl6ZSBleGNyZXRpb24sIGFuZCB0aGUgaW1wbGVtZW50YXRpb24gb2YgYmVzdCBtYW5hZ2VtZW50IHByYWN0aWNlcyB0byBwcm9tb3RlIHN1c3RhaW5hYmxlIHJ1bWluYW50IHByb2R1Y3Rpb24uIEJ5IGludGVncmF0aW5nIHNjaWVudGlmaWMgZmluZGluZ3Mgd2l0aCBwcmFjdGljYWwgYXBwbGljYXRpb25zLCB0aGlzIHJldmlldyB1bmRlcnNjb3JlcyB0aGUgaW1wb3J0YW5jZSBvZiBhY2hpZXZpbmcgaGFybW9ueSBiZXR3ZWVuIG51dHJpdGlvbiBvcHRpbWl6YXRpb24gYW5kIGVudmlyb25tZW50YWwgcmVzcG9uc2liaWxpdHkgaW4gbW9kZXJuIGFncmljdWx0dXJlLiBUaHJvdWdoIHRoaXMgY29tcHJlaGVuc2l2ZSBhbmFseXNpcywgdGhlIGNoYXB0ZXIgYWltcyBmb3IgYmFsYW5jaW5nIHRoZSBkdWFsIGdvYWxzIG9mIG9wdGltaXppbmcgcnVtaW5hbnQgbnV0cml0aW9uIGFuZCBtaW5pbWl6aW5nIGVudmlyb25tZW50YWwgaW1wYWN0LCB0aGVyZWJ5IGNvbnRyaWJ1dGluZyB0byBtb3JlIHN1c3RhaW5hYmxlIGFuZCByZXNpbGllbnQgYWdyaWN1bHR1cmFsIHN5c3RlbXMuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjU3NzIvaW50ZWNob3Blbi4xMDA2NTc4IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5pbnRlY2hvcGVuLmNvbS9jaXRhdGlvbi1wZGYtdXJsLzExODc4MTQiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMS0xNSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IlZldGVyaW5hcnkgbWVkaWNpbmUgYW5kIHNjaWVuY2UiLCJpbXBhY3RGYWN0b3IiOiIwLjk5In0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkludGVjaE9wZW4ifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjUyLCJjaXRlZEJ5Q291bnQiOjF9fSwicXVvdGUiOiJXYXRlciBRdWFsaXR5IENoYW5nZXMg4oCiIENsYXJpdHkgYW5kIEFsZ2FsIEJsb29tczogV2F0ZXIgY2xhcml0eSBtYXkgZGVjcmVhc2Ugc2xpZ2h0bHkgZHVlIHRvIHRoZSBpbml0aWFsIGFsZ2FsIGJsb29tcy4gSG93ZXZlciwgdGhpcyBwaGFzZSBpcyBvZnRlbiBzdGlsbCBjaGFyYWN0ZXJpemVkIGJ5IHJlbGF0aXZlbHkgZ29vZCB3YXRlciBxdWFsaXR5LiDigKIgQmlvZGl2ZXJzaXR5OiBUaGUgZWNvc3lzdGVtIG1pZ2h0IHN0aWxsIHN1cHBvcnQgYSBkaXZlcnNlIHJhbmdlIG9mIGFxdWF0aWMgb3JnYW5pc21zLCBpbmNsdWRpbmcgZmlzaCBhbmQgaW52ZXJ0ZWJyYXRlcy4gMy4xLjQuMiBQaGFzZSAyOiBNZXNvdHJvcGhpYyB0byBldXRyb3BoaWMgdHJhbnNpdGlvbiAxLiBOdXRyaWVudCBJbWJhbGFuY2Ug4oCiIFBob3NwaG9ydXMgdnMuIE5pdHJvZ2VuOiBBcyBhZ3JpY3VsdHVyYWwgcHJhY3RpY2VzIGludGVuc2lmeSBhbmQgdXJiYW5pemF0aW9uIGluY3JlYXNlcywgcGhvc3Bob3J1cyBpbnB1dHMgKGUuZy4sIGZyb20gZmVydGlsaXplcnMgYW5kIGRldGVyZ2VudHMpIG9mdGVuIGV4Y2VlZCB0aG9zZSBvZiBuaXRyb2dlbi4gVGhpcyBpbWJhbGFuY2UgY2FuIGRyaXZlIHNpZ25pZmljYW50IGNoYW5nZXMgaW4gd2F0ZXIgY2hlbWlzdHJ5IGFuZCBlY29zeXN0ZW0gZHluYW1pY3MuIDIuIEFsZ2FsIEJsb29tIEludGVuc2lmaWNhdGlvbiDigKIgQ3lhbm9iYWN0ZXJpYSBEb21pbmFuY2U6IEV4Y2VzcyBwaG9zcGhvcnVzIGNhbiBsZWFkIHRvIHRoZSBkb21pbmFuY2Ugb2YgY3lhbm9iYWN0ZXJpYSAoYmx1ZS1ncmVlbiBhbGdhZSksIHdoaWNoIGNhbiBvdXRjb21wZXRlIG90aGVyIGFsZ2FlIGFuZCBhcXVhdGljIHBsYW50cy4gVGhlc2UgYmxvb21zIGNhbiBiZSB0b3hpYyBhbmQgaGFybWZ1bCB0byBhcXVhdGljIGxpZmUuIOKAoiBIeXBveGlhOiBUaGUgZGVjb21wb3NpdGlvbiBvZiBsYXJnZSBhbGdhbCBibG9vbXMgY29uc3VtZXMgb3h5Z2VuLCBsZWFkaW5nIHRvIGh5cG94aWMgKGxvdyBveHlnZW4pIG9yIGFub3hpYyAobm8gb3h5Z2VuKSBjb25kaXRpb25zLCBwYXJ0aWN1bGFybHkgaW4gZGVlcGVyIHdhdGVycy4gMy4gV2F0ZXIgUXVhbGl0eSBEZXRlcmlvcmF0aW9uIOKAoiBEZWNyZWFzZWQgQ2xhcml0eTogV2F0ZXIgY2xhcml0eSBzaWduaWZpY2FudGx5IGRlY3JlYXNlcyBkdWUgdG8gZGVuc2UgYWxnYWwgZ3Jvd3RoLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo2Ny4yNjEwMDE1ODY5MTQwNiwibGVmdCI6MTUuNTgzNDU4OTAwNDUxNjYsIndpZHRoIjoyMC40NjA2MTg5NzI3NzgzMiwiaGVpZ2h0IjoxLjkwNTE0ODYyNTM3Mzg0MDN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NzAuNTEzMDMxMDA1ODU5MzgsImxlZnQiOjE3LjM5ODIxNjI0NzU1ODU5NCwid2lkdGgiOjY2LjQwMzU0OTE5NDMzNTk0LCJoZWlnaHQiOjEuOTA1MTU3MDg5MjMzMzk4NH19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo3Mi4xMzkwNDU3MTUzMzIwMywibGVmdCI6MTkuMDg4MzU2MDE4MDY2NDA2LCJ3aWR0aCI6NjUuNzg4NTIwODEyOTg4MjgsImhlaWdodCI6My41Mjg0NjA1MDI2MjQ1MTE3fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjc3LjAxNzA5NzQ3MzE0NDUzLCJsZWZ0IjoxNy4zOTgyMTYyNDc1NTg1OTQsIndpZHRoIjo2Mi4wOTQ1NzAxNTk5MTIxMSwiaGVpZ2h0IjoxLjkwNTE1NzA4OTIzMzM5ODR9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NzguNjQzMTEyMTgyNjE3MTksImxlZnQiOjE5LjA4ODM1NjAxODA2NjQwNiwid2lkdGgiOjM2LjcyNTMxMTI3OTI5Njg3NSwiaGVpZ2h0IjoxLjkwMjQ0NDI0MzQzMTA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6ODEuODk1MTMzOTcyMTY3OTcsImxlZnQiOjEzLjI0Njg1OTU1MDQ3NjA3NCwid2lkdGgiOjQwLjM5MTM5NTU2ODg0NzY1NiwiaGVpZ2h0IjoxLjkwNTE1NzA4OTIzMzM5ODR9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6ODUuMTQ3MTYzMzkxMTEzMjgsImxlZnQiOjE1LjU4MzQ1ODkwMDQ1MTY2LCJ3aWR0aCI6MTcuNjAyMTc4NTczNjA4NCwiaGVpZ2h0IjoxLjkwNTE1NzA4OTIzMzM5ODR9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6ODguMzk5MTkyODEwMDU4NiwibGVmdCI6MTcuMzk4MjE2MjQ3NTU4NTk0LCJ3aWR0aCI6NjUuMDA3MzQ3MTA2OTMzNiwiaGVpZ2h0IjoxLjkwNTE1NzA4OTIzMzM5ODR9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6OTAuMDI1MjA3NTE5NTMxMjUsImxlZnQiOjE5LjA4ODM1NjAxODA2NjQwNiwid2lkdGgiOjYxLjA1MTA1MjA5MzUwNTg2LCJoZWlnaHQiOjEuOTAyNDQ0MjQzNDMxMDkxM319LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjoxMC4zNTA0MTcxMzcxNDU5OTYsImxlZnQiOjIwLjg0MDgwNTA1MzcxMDkzOCwid2lkdGgiOjY2LjI3NzA1MzgzMzAwNzgxLCJoZWlnaHQiOjMuNTI4NDU1MjU3NDE1NzcxNX19LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjoxNS40OTk0Njc4NDk3MzE0NDUsImxlZnQiOjE3LjMzNTkwNjk4MjQyMTg3NSwid2lkdGgiOjIzLjUxOTYxNTE3MzMzOTg0NCwiaGVpZ2h0IjoxLjkwNTE0ODYyNTM3Mzg0MDN9fSx7InBhZ2VOdW1iZXIiOjEzLCJiYm94Ijp7InRvcCI6MTguNzUxNTAxMDgzMzc0MDIzLCJsZWZ0IjoxOS4xNTA2NjUyODMyMDMxMjUsIndpZHRoIjo2NC40Mzg3NjY0Nzk0OTIxOSwiaGVpZ2h0IjoxLjkwNTE0ODYyNTM3Mzg0MDN9fSx7InBhZ2VOdW1iZXIiOjEzLCJiYm94Ijp7InRvcCI6MjAuMzc3NTE3NzAwMTk1MzEyLCJsZWZ0IjoyMC44NDA4MDUwNTM3MTA5MzgsIndpZHRoIjo2MS4wNzQ0MjQ3NDM2NTIzNDQsImhlaWdodCI6My41Mjg0NTQ1NDIxNjAwMzR9fSx7InBhZ2VOdW1iZXIiOjEzLCJiYm94Ijp7InRvcCI6MjUuMjU1NTY3NTUwNjU5MTgsImxlZnQiOjE5LjE1MDY2NTI4MzIwMzEyNSwid2lkdGgiOjY4LjAyMzU1OTU3MDMxMjUsImhlaWdodCI6MS45MDUxNDg2MjUzNzM4NDAzfX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjI2Ljg4MTU4MjI2MDEzMTgzNiwibGVmdCI6MjAuODQwODA1MDUzNzEwOTM4LCJ3aWR0aCI6NjYuMDc2MjkzOTQ1MzEyNSwiaGVpZ2h0IjozLjUyODQ3MDk5MzA0MTk5Mn19LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjozMi4wMzA2NTEwOTI1MjkzLCJsZWZ0IjoxNy4zMzU5MDY5ODI0MjE4NzUsIndpZHRoIjoyNC42Nzc5NzY2MDgyNzYzNjcsImhlaWdodCI6MS45MDUxNDg2MjUzNzM4NDAzfX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjM1LjI4MjY4NDMyNjE3MTg3NSwibGVmdCI6MTkuMTUwNjY1MjgzMjAzMTI1LCJ3aWR0aCI6NjIuNDE5Nzg4MzYwNTk1NywiaGVpZ2h0IjoxLjkwNTE0ODYyNTM3Mzg0MDN9fSx7InBhZ2VOdW1iZXIiOjEzLCJiYm94Ijp7InRvcCI6MzYuOTA4Njk5MDM1NjQ0NTMsImxlZnQiOjIwLjg0MDgwNTA1MzcxMDkzOCwid2lkdGgiOjYuNjMzOTkzMTQ4ODAzNzExLCJoZWlnaHQiOjEuOTAyNDQwMTkwMzE1MjQ2Nn19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEzIn19XQ%3D%3D. Massive fish kills, documented across Indian lakes, underscore this peril: in Rewalsar Lake, warming-driven eutrophication depleted DO, triggering biodiversity crashes and toxin surges (Kumar & Mahajan, 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlBhd2FuIiwiZmFtaWx5IjoiS3VtYXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW1icmlzaCBLdW1hciIsImZhbWlseSI6Ik1haGFqYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyNTIzLTM5NjMiLCIyNTIzLTM5NzEiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMDcvczQyNDUyLTAyMC0zMDgyLTgiLCJtYWciOiIzMDM1OTYwMjI4Iiwib3BlbmFsZXgiOiJXMzAzNTk2MDIyOCJ9LCJ0aXRsZSI6IlRyb3BoaWMgc3RhdHVzIGFuZCBpdHMgcmVndWxhdGluZyBmYWN0b3IgZGV0ZXJtaW5hdGlvbiBhdCB0aGUgUmV3YWxzYXIgTGFrZSwgbm9ydGh3ZXN0IEhpbWFsYXlhIChIUCksIEluZGlhLCBiYXNlZCBvbiBzZWxlY3RlZCBwYXJhbWV0ZXJzIGFuZCBtdWx0aXZhcmlhdGUgc3RhdGlzdGljYWwgYW5hbHlzaXMiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAwNy9zNDI0NTItMDIwLTMwODItOCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9saW5rLnNwcmluZ2VyLmNvbS9jb250ZW50L3BkZi8xMC4xMDA3L3M0MjQ1Mi0wMjAtMzA4Mi04LnBkZiIsImlzc3VlZERhdGUiOiIyMDIwLTA2LTIyIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiU04gQXBwbGllZCBTY2llbmNlcyIsImlzc3VlIjo3LCJ2b2x1bWUiOjIsImltcGFjdEZhY3RvciI6IjcuMzUifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiU3ByaW5nZXIgTmF0dXJlIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo0MCwiY2l0ZWRCeUNvdW50IjoxOX19LCJxdW90ZSI6IkNoYW5naW5nIHF1YWxpdHkgaGFzIGNyZWF0ZWQgdGhlIHdhdGVyIHNjYXJjaXR5IGFuZCAgYSBodWdlIG1hc3Mgb2YgcG9wdWxhdGlvbiB0aHVzIGJlZW4gZGVwcml2ZWQgZnJvbSB1c2luZyAgdGhlIGZyZXNod2F0ZXIgZm9yIHZhcmlvdXMgcHVycG9zZXMgWzMsIDRdLiBIb3dldmVyLCB0aGUgIG9ibm94aW91cyBtYXRlcmlhbHMgdGhhdCBzdGF5IGluIHdhdGVyIGJvZGllcyBmb3IgYSBsb25nLXRpbWUgcGVyaW9kIGJyaW5nIGNoYW5nZXMgaW4gdGhlIHBoeXNpb2NoZW1pY2FsIGNoYXItYWN0ZXJpc3RpY3MgYXMgd2VsbCBhcyB0aGUgbmF0dXJhbCBzY2VuYXJpbyBvZiB0aGUgbGFrZXMgWzVdLiAgVGhlIEhpbWFsYXlhbiBsYWtlcyBhcmUgdW5kZXIgc3RyZXNzIGR1ZSB0byBtaXNtYW5hZ2VkIGh1bWFuIGFjdGl2aXRpZXMsIHdoaWNoIGluY3JlYXNlIG51dHJpZW50cyBhbmQgcG9sbHV0aW9uIGluIHRlcm1zIG9mIGhlYXZ5IG1ldGFscyB0b3hpY2l0eSBhbmQgYmFjdGVyaWFsIGNvbnRhbWluYXRpb24gIFs24oCTOF0uIFRoZSBtYW5hZ2VtZW50IGFuZCBwcmVzZXJ2YXRpb24gb2YgdGhlIGxha2VzIGF0ICBwcmVzZW50IGhhdmUgYmVjb21lIGEgZ3JlYXQgY2hhbGxlbmdlIGZvciBlbnZpcm9ubWVudGFsLWlzdHMgYW5kIG1hbmFnZXJzIFs1LCA5XS4gVGhlIGV1dHJvcGhpY2F0aW9uIGlzIGFsc28gYSBtYWluIHByb2JsZW0gb2YgdGhlICBmcmVzaHdhdGVyIGxha2VzLCBhbmQgaXQgaXMgc3BlY2lmaWVkIGZvciB0aGUgbGFrZXMgaGF2aW5nICBudXRyaWVudCBlbnJpY2htZW50LiBJdCBjYXVzZXMgZXhjZXNzaXZlIGdyb3d0aCBvZiBhbGdhZSAgYW5kIGlzIGNvbnNpZGVyZWQgdG8gYmUgcmVzcG9uc2libGUgZm9yIGRlZ3JhZGluZyB0aGUgIHF1YWxpdGF0aXZlIG5hdHVyZSBvZiB0aGUgbGFrZXMgWzUsIDEwXS4gVGhlIGV1dHJvcGhpY2F0aW9uICBpcyBmb2xsb3dlZCBieSB0aGUgYWxnYWwgYmxvb20sIHdoaWNoIGdlbmVyYXRlcyB0b3hpbiwgIHdvcnNlbnMgd2F0ZXIgcXVhbGl0eSwgZGVwbGV0ZXMgZGlzc29sdmVkIG94eWdlbiAoRE8pICogUGF3YW4gS3VtYXIsIHBhd2FuLmV2c0BnbWFpbC5jb20gfCAxRGVwYXJ0bWVudCBvZiBFbnZpcm9ubWVudGFsIFNjaWVuY2VzLCBDZW50cmFsIFVuaXZlcnNpdHkgb2YgSGltYWNoYWwgUHJhZGVzaCwgRGhhcmFtc2hhbGEgMTc2MjE1LCBJbmRpYS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2OC41Nzg0NjA2OTMzNTkzOCwibGVmdCI6OC41NzE0MTg3NjIyMDcwMzEsIndpZHRoIjo0MC4zMjc1MTQ2NDg0Mzc1LCJoZWlnaHQiOjEuNTE5ODU1MjYwODQ4OTk5fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NzAuMTU4OTk2NTgyMDMxMjUsImxlZnQiOjguNTY5NzM4Mzg4MDYxNTIzLCJ3aWR0aCI6NDAuMzU0NzI0ODg0MDMzMiwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjcxLjczOTU0NzcyOTQ5MjE5LCJsZWZ0Ijo4LjU2OTczODM4ODA2MTUyMywid2lkdGgiOjQwLjM0OTY5MzI5ODMzOTg0NCwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjczLjMyMDA5MTI0NzU1ODYsImxlZnQiOjguNTY4MDU5OTIxMjY0NjQ4LCJ3aWR0aCI6NDAuMDIwMDg4MTk1ODAwNzgsImhlaWdodCI6MS41MTk4NTUyNjA4NDg5OTl9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo3NC45MDA2NDIzOTUwMTk1MywibGVmdCI6OC41NjgwNTk5MjEyNjQ2NDgsIndpZHRoIjozOS45OTk5Mjc1MjA3NTE5NSwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjc2LjQ4MTE4NTkxMzA4NTk0LCJsZWZ0Ijo4LjU2ODA1NzA2MDI0MTcsIndpZHRoIjo0MC4zNTcwNzQ3Mzc1NDg4MywiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjc4LjA2MTcyOTQzMTE1MjM0LCJsZWZ0Ijo4LjU2OTczNjQ4MDcxMjg5LCJ3aWR0aCI6NDAuMzUzNzEzOTg5MjU3ODEsImhlaWdodCI6MS41MTk4NTUyNjA4NDg5OTl9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2MC42NzU3Mjc4NDQyMzgyOCwibGVmdCI6NTEuNDI3MTY1OTg1MTA3NDIsIndpZHRoIjo0MC4zNzg1NjY3NDE5NDMzNiwiaGVpZ2h0IjoxOC45MDU4NTUxNzg4MzMwMDh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo4MS45MjczMzc2NDY0ODQzOCwibGVmdCI6OC40MDE0OTc4NDA4ODEzNDgsIndpZHRoIjo3My45NTQ2OTY2NTUyNzM0NCwiaGVpZ2h0IjoyLjY3ODc2MjY3NDMzMTY2NX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEifX1d. Chilika's fisheries have seen yields plummet by 40% amid recurrent dead zones, with scum layers exacerbating gill clogging and osmoregulatory failure in endemic species (Jilbert et al., 2020; Nair & Nayak, 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRvbSIsImZhbWlseSI6IkppbGJlcnQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUmFvdWzigJBNYXJpZSIsImZhbWlseSI6IkNvdXR1cmUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQnJpYW4iLCJmYW1pbHkiOiJIdXNlciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLYWxldmkiLCJmYW1pbHkiOiJTYWxvbmVuIn19XSwiaWRzIjp7Imlzc24iOlsiMDAxOC04MTU4IiwiMTU3My01MTE3Il0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDA3L3MxMDc1MC0wMjAtMDQ0NTcteCIsIm1hZyI6IjMxMDA0NTkwNTAiLCJvcGVuYWxleCI6IlczMTAwNDU5MDUwIn0sInRpdGxlIjoiUHJlZmFjZTogUmVzdG9yYXRpb24gb2YgZXV0cm9waGljIGxha2VzOiBjdXJyZW50IHByYWN0aWNlcyBhbmQgZnV0dXJlIGNoYWxsZW5nZXMiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAwNy9zMTA3NTAtMDIwLTA0NDU3LXgiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vbGluay5zcHJpbmdlci5jb20vY29udGVudC9wZGYvMTAuMTAwNy9zMTA3NTAtMDIwLTA0NDU3LXgucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjAtMTEtMTYiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA3IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJIeWRyb2Jpb2xvZ2lhIiwiaXNzdWUiOjIxLCJ2b2x1bWUiOjg0NywiaW1wYWN0RmFjdG9yIjoiMy4wMCJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjQzNDMsImxhc3RQYWdlIjo0MzU3fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlNwcmluZ2VyIFNjaWVuY2UrQnVzaW5lc3MgTWVkaWEifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjk1LCJjaXRlZEJ5Q291bnQiOjU4fX0sInF1b3RlIjoiU2ltaWxhciBkZWdyYWRhdGlvbiBvZiBmcmVzaHdhdGVyIGVjb3N5c3RlbXMgaXMgb2JzZXJ2ZWQgd29ybGR3aWRlLCB3aXRoIHRoZSB0aW1lbGluZSBvZiBodW1hbiBwcmVzc3VyZSBkaWN0YXRlZCBieSB0cmVuZHMgaW4gdXJiYW5pemF0aW9uIGFuZCBpbnRlbnNpdmUgYWdyaWN1bHR1cmUgaW4gZGlmZmVyZW50IGdlb2dyYXBoaWMgcmVnaW9ucyAoZS5nLiwgVmVubmVtbyBldCBhbC4sIDIwMDkpLiBUaGUgcHJpbmNpcGFsIGRyaXZlciBvZiBlY29zeXN0ZW0gc3RyZXNzIGluIG1vc3QgYXF1YXRpYyBzeXN0ZW1zIGlzIGV1dHJvcGhpY2F0aW9uLCB0aGUgZXhjZXNzIGdyb3d0aCBvZiBhbGdhZSBkcml2ZW4gYnkgYW50aHJvcG9nZW5pYyBpbnB1dHMgb2YgdGhlIG1hY3JvbnV0cmllbnRzIG5pdHJvZ2VuIChOKSBhbmQgcGhvc3Bob3J1cyAoUCkgKFNtaXRoLCAyMDAzOyBCaXJrIGV0IGFsLiwgMjAyMCkuIE51dHJpZW50cyBhcmUgZW1pdHRlZCBmcm9tIGJvdGggcG9pbnQgYW5kIGRpZmZ1c2Ugc291cmNlcyBpbiB0aGUgaHVtYW4gZW52aXJvbm1lbnQgYW5kIGVudGVyIGFxdWF0aWMgc3lzdGVtcyB2aWEgc3VyZmFjZSBydW5vZmYsIGFzIHdlbGwgYXMgZ3JvdW5kLXdhdGVyIGFuZCBhdG1vc3BoZXJpYyBkZXBvc2l0aW9uIChQYWVybCwgMTk5NykuIEV1dHJvcGhpY2F0aW9uIG9mdGVuIGxlYWRzIHRvIHRoZSBkb21pbmFuY2Ugb2YgcHJpbWFyeSBwcm9kdWN0aW9uIGJ5IGN5YW5vYmFjdGVyaWEsIGluY2x1ZGluZyB0b3hpYyBnZW5lcmEgc3VjaCBhcyBEb2xpY2hvc3Blcm11bSwgQXBoYW5pem9tZW5vbiwgYW5kIE1pY3JvY3lzdGlzIChIdWlzbWFuIGV0IGFsLiwgMjAxOCksIGFzIHdlbGwgYXMgb3h5Z2VuIGRlcGxldGlvbiBhbmQgaGFiaXRhdCBkZWdyYWRhdGlvbiBpbiBsYWtlcyAoQXJlbmQgZXQgYWwuLCAyMDExKS4gQmVjYXVzZSBsYWtlcyBwcm92aWRlIGEgcmFuZ2Ugb2YgY3J1Y2lhbCBlY29zeXN0ZW0gc2VydmljZXMgKFdpbHNvbiAmIENhcnBlbnRlciwgMTk5OSksIHRoZXJlIGlzIGEgZ3Jvd2luZyBuZWVkIGZvciBtYW5hZ2VtZW50IGFwcHJvYWNoZXMgdGhhdCByZXN0b3JlIHdhdGVyIHF1YWxpdHkgaW4gZnJlc2h3YXRlciBzeXN0ZW1zLiBXaXRoaW4gdGhlIGZyYW1ld29yayBvZiBVbml0ZWQgTmF0aW9ucyBTdXN0YWluYWJsZSBEZXZlbG9wLW1lbnQgR29hbCA2IChDbGVhbiBXYXRlciBhbmQgU2FuaXRhdGlvbiksIHNldmVyYWwgZGlzdGluY3QgdGFyZ2V0cyByZWxhdGUgdG8gdGhlIGltcHJvdmVtZW50IG9mIHdhdGVyIHF1YWxpdHkgYW5kIHByb3RlY3Rpb24gb2YgYXF1YXRpYyBlY29zeXN0ZW1zIGJ5IDIwMzAgKFVuaXRlZCBOYXRpb25zLCAyMDE1KS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyOC4yOTg4NjQzNjQ2MjQwMjMsImxlZnQiOjguNTQ5MjA1NzgwMDI5Mjk3LCJ3aWR0aCI6MzkuMzcxNDk0MjkzMjEyODksImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MjkuOTkxMTM0NjQzNTU0Njg4LCJsZWZ0Ijo4LjU0OTIwNTc4MDAyOTI5Nywid2lkdGgiOjM5LjM2NTExOTkzNDA4MjAzLCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMxLjY4MzQwNjgyOTgzMzk4NCwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNjk2NjMyMzg1MjUzOSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozMy4zNjc5NzMzMjc2MzY3MiwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNzU2NzkwMTYxMTMyOCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNS4wNjAyNDkzMjg2MTMyOCwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNzA5NjQwNTAyOTI5NywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNi43NTI1MTc3MDAxOTUzMSwibGVmdCI6OC41NDkyMDg2NDEwNTIyNDYsIndpZHRoIjozOS4zNzY5NjA3NTQzOTQ1MywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC40MzcwODgwMTI2OTUzMSwibGVmdCI6OC41NDkyMDg2NDEwNTIyNDYsIndpZHRoIjozOS4zNjY1Njk1MTkwNDI5NywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0MC4xMjkzNTYzODQyNzczNDQsImxlZnQiOjguNTQ5MjA4NjQxMDUyMjQ2LCJ3aWR0aCI6MzkuMzY0Mzc5ODgyODEyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0MS44MjE2MjQ3NTU4NTkzNzUsImxlZnQiOjguNTQ5MjA4NjQxMDUyMjQ2LCJ3aWR0aCI6MzkuMzY1NjczMDY1MTg1NTUsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDMuNTA2MTk1MDY4MzU5Mzc1LCJsZWZ0Ijo4LjU0OTM4OTgzOTE3MjM2Mywid2lkdGgiOjM5LjM3MDM5MTg0NTcwMzEyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0NS4xOTg0NjM0Mzk5NDE0MDYsImxlZnQiOjguNTQ5NTcyOTQ0NjQxMTEzLCJ3aWR0aCI6MzkuMzY1ODQ0NzI2NTYyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ni44OTA3MzU2MjYyMjA3LCJsZWZ0Ijo4LjU0OTU3Mjk0NDY0MTExMywid2lkdGgiOjM5LjM3Mjk1NTMyMjI2NTYyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0OC41NzUzMDIxMjQwMjM0NCwibGVmdCI6OC41NDk1NzI5NDQ2NDExMTMsIndpZHRoIjozOS4zNzI0MjEyNjQ2NDg0NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1MC4yNjc1NzgxMjUsImxlZnQiOjguNTQ5NTc2NzU5MzM4Mzc5LCJ3aWR0aCI6MzkuMzcxMTI0MjY3NTc4MTI1LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjUxLjk1OTg0MjY4MTg4NDc2NiwibGVmdCI6OC41NDk1NzY3NTkzMzgzNzksIndpZHRoIjozOS4zNjc4MzU5OTg1MzUxNTYsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTMuNTc0MTE5NTY3ODcxMDk0LCJsZWZ0Ijo4LjU0OTU3Njc1OTMzODM3OSwid2lkdGgiOjM5LjM3MDc3NzEzMDEyNjk1LCJoZWlnaHQiOjEuMzM2ODk1NTg1MDYwMTE5Nn19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjU1LjI2NjM5MTc1NDE1MDM5LCJsZWZ0Ijo4LjU0OTc2MDgxODQ4MTQ0NSwid2lkdGgiOjM5LjM2NjU4MDk2MzEzNDc2NiwiaGVpZ2h0IjoxLjMzNjg5NTU4NTA2MDExOTZ9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1Ny4wMjg5NTM1NTIyNDYwOTQsImxlZnQiOjguNTQ5OTQ0ODc3NjI0NTEyLCJ3aWR0aCI6MzkuMzYyMDI2MjE0NTk5NjEsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTguNzEzNTIzODY0NzQ2MDk0LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjkuMjk5OTgyMDcwOTIyODUyLCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjYwLjQwNTc5MjIzNjMyODEyNSwibGVmdCI6MTAuNjIyNDU1NTk2OTIzODI4LCJ3aWR0aCI6MzcuMjg3ODY4NDk5NzU1ODYsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NjIuMDk4MDYwNjA3OTEwMTU2LCJsZWZ0Ijo4LjU0OTk0MzkyMzk1MDE5NSwid2lkdGgiOjM5LjM3MzMyNTM0NzkwMDM5LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjYzLjc4MjYzMDkyMDQxMDE1NiwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzMxMjY5ODM2NDI1OCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2NS40NzQ5MDY5MjEzODY3MiwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNjUxMTk5MzQwODIwMywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2Ny4xNjcxNzUyOTI5Njg3NSwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNjk0ODAxMzMwNTY2NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2OC43ODE0NDgzNjQyNTc4MSwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzczMzg0MDk0MjM4MywiaGVpZ2h0IjoxLjMzNjg5MTUzMTk0NDI3NX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjcwLjU0NDAxMzk3NzA1MDc4LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjM5LjM2OTMwODQ3MTY3OTY5LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjcyLjIzNjI4OTk3ODAyNzM0LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjM5LjM3MjAzMjE2NTUyNzM0NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo3My45MjA4NDUwMzE3MzgyOCwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzEzMDczNzMwNDY4NzUsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNDM0NCJ9fSx7Im9yaWdpbiI6NCwid29yayI6eyJ0eXBlIjo4LCJzdWJ0eXBlIjp7Im9uZW9mS2luZCI6ImFydGljbGVUeXBlIiwiYXJ0aWNsZVR5cGUiOjF9LCJjb250cmlidXRvcnMiOlt7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJOYXZ5YSBWaWtyYW1hbiIsImZhbWlseSI6Ik5haXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUHJhdGVlcCBLdW1hciIsImZhbWlseSI6Ik5heWFrIn19XSwiaWRzIjp7Imlzc24iOlsiMjI5Ni03NzQ1Il0sImlzYm4iOltdLCJkb2kiOiIxMC4zMzg5L2ZtYXJzLjIwMjMuMTA4NzI5NiIsIm9wZW5hbGV4IjoiVzQzODI3MjMxMzMifSwidGl0bGUiOiJVbmNvdmVyaW5nIHdhdGVyIHF1YWxpdHkgYW5kIGV2YWx1YXRpbmcgdnVsbmVyYWJpbGl0aWVzIG9mIHNtYWxsLXNjYWxlIGZpc2hlcmllcyBpbiBDaGlsaWthIExhZ29vbiwgSW5kaWEiLCJhYnN0cmFjdCI6IlNtYWxsLXNjYWxlIGZpc2hlcmllcyAoU1NGcykgcGxheSBhIHN0cm9uZyByb2xlIGluIHN1c3RhaW5pbmcgbWlsbGlvbnMgb2YgbGl2ZWxpaG9vZHMsIGZvb2Qgc2VjdXJpdHksIG51dHJpdGlvbiwgYW5kIGluY29tZSBnbG9iYWxseSBidXQgdGhlIGZpc2hlcnMgZW5nYWdlZCBpbiB0aGlzIHNlY3RvciBzaW11bHRhbmVvdXNseSBleHBlcmllbmNlIGhpZ2ggbGV2ZWxzIG9mIHZ1bG5lcmFiaWxpdHkgYW5kIHByb2Nlc3NlcyBvZiBtYXJnaW5hbGlzYXRpb24uIFNldmVyYWwgZmFjdG9ycyBhcmUgYXR0cmlidXRlZCB0byB0aGUgbXVsdGlkaW1lbnNpb25hbCB2dWxuZXJhYmlsaXRpZXMgdGhlIHNtYWxsLXNjYWxlIGZpc2hlcnMgZXhwZXJpZW5jZSwgc3Bhbm5pbmcgYm90aCBuYXR1cmFsIChlLmcuLCBuYXR1cmFsIGRpc2FzdGVycywgZWNvc3lzdGVtIGNoYW5nZSkgYW5kIGFudGhyb3BvZ2VuaWMgKGUuZy4sIHBvbGljeSBjaGFuZ2UsIGh5ZHJvbG9naWNhbCBpbnRlcnZlbnRpb25zLCBhcXVhY3VsdHVyZSkgcHJlc3N1cmVzLiBXaGlsZSB0aGVyZSBpcyBtdWNoIGxpdGVyYXR1cmUgb24gdmFyaW91cyBuYXR1cmFsIGFuZCBodW1hbiBkcml2ZXJzIG9mIHZ1bG5lcmFiaWxpdHkgaW4gc21hbGwtc2NhbGUgZmlzaGVyeSBjb21tdW5pdGllcywgYW4gYWJzZW5jZSBvZiByZXNlYXJjaCBjb25uZWN0aW5nIHZ1bG5lcmFiaWxpdHkgd2l0aCB3YXRlciBxdWFsaXR5IGlzIGV2aWRlbnQuIEZpc2hlciBjb21tdW5pdGllcyBvZnRlbiB0YWxrIGFib3V0IGZpc2ggaW4gcmVsYXRpb24gdG8gdGhlIGhlYWx0aCBvZiB0aGVpciBhcXVhdGljIGhhYml0YXRzIHdoZXJlaW4gd2F0ZXIgcXVhbGl0eSBpcyBzZWVuIGFzIGEga2V5IHBhcmFtZXRlci4gVGhlIGxpbmsgYmV0d2VlbiBoZWFsdGh5IGZpc2ggYW5kIGdvb2QgcXVhbGl0eSB3YXRlciBoYXMgc2lnbmlmaWNhbnQgaW1wbGljYXRpb25zIGZvciBzdHJvbmcgYW5kIHZpYWJsZSBmaXNoaW5nIGNvbW11bml0aWVzLiBUaGlzIHBhcGVyIGV4YW1pbmVzIHRoZXNlIGxpbmtzIGZ1cnRoZXIgYnkgZm9jdXNpbmcgb24gdGhlIG5hdHVyZSBvZiB2dWxuZXJhYmlsaXRpZXMgY2F1c2VkIGJ5IHdhdGVyIHF1YWxpdHkgY2hhbmdlcyBpbiB0aGUgc21hbGwtc2NhbGUgZmlzaGVyeSBzeXN0ZW0gb2YgQ2hpbGlrYSBMYWdvb24gaW4gSW5kaWEuIFdlIHVuZGVydGFrZSBkZXRhaWxlZCBhbmFseXNpcyBvZiB0aGUgaW52YXNpdmUgc2hyaW1wIGFxdWFjdWx0dXJlIGFjdGl2aXRpZXMgYW5kIGh5ZHJvbG9naWNhbCBpbnRlcnZlbnRpb25zIGZvciBvcGVuaW5nIG9mIGEgbGFnb29uIGlubGV0IHdpdGggdGhlIEJheSBvZiBCZW5nYWwgYXMgdHdvIGRvbWluYW50IGRyaXZlcnMgYWR2ZXJzZWx5IGltcGFjdGluZyB3YXRlciBxdWFsaXR5IGFuZCBpbmNyZWFzaW5nIHZ1bG5lcmFiaWxpdGllcyBvZiB0aGUgZW50aXJlIHNtYWxsLXNjYWxlIGZpc2hlcmllcyBzb2NpYWwtZWNvbG9naWNhbCBzeXN0ZW0uIE91ciBhbmFseXNpcyBzdWdnZXN0cyB0aGF0IHRoZXJlIGFyZSBzdHJvbmcgaW50ZXJjb25uZWN0aW9ucyBiZXR3ZWVuIGNoYW5nZXMgaW4gd2F0ZXIgcXVhbGl0eSBhbmQgdGhlIGxldmVscyBvZiB2dWxuZXJhYmlsaXRpZXMgaW4gdGhlIFNTRnMgb2YgQ2hpbGlrYSBMYWdvb24uIFBvbGx1dGFudHMgc3VjaCBhcyBwZXN0aWNpZGVzLCBhbmQgb3JnYW5pYyBjb21wb3VuZHMgYWNjdW11bGF0ZSBpbiBmaXNoIHRpc3N1ZXMgYW5kIGFmZmVjdCB0aGVpciBncm93dGgsIHJlcHJvZHVjdGlvbiwgYW5kIG92ZXJhbGwgaGVhbHRoLiBUaGlzIGxlZCB0byBkZWNsaW5lcyBpbiBmaXNoIHBvcHVsYXRpb25zLCBtYWtpbmcgaXQgbW9yZSBkaWZmaWN1bHQgZm9yIGZpc2hlcnMgdG8gbWFrZSBhIGxpdmluZy4gSW4gYWRkaXRpb24gdG8gZGlyZWN0IGltcGFjdHMgb24gZmlzaCBwb3B1bGF0aW9ucywgcG9vciB3YXRlciBxdWFsaXR5IGFsc28gaGFzIGluZGlyZWN0IGVmZmVjdHMgb24gdGhlIHNvY2lhbCBhbmQgZWNvbm9taWMgdnVsbmVyYWJpbGl0eSBvZiBTU0ZzLiBGb3IgZXhhbXBsZSwgY29udGFtaW5hdGlvbiBvZiB3YXRlciBzb3VyY2VzIGxlZCB0byB0aGUgcmVkdWNlZCBudW1iZXIgb2YgZmlzaCBzcGVjaWVzIHJlZHVjaW5nIHRoZSBhbW91bnQgb2YgdGltZSBmaXNoZXJzIGNhbiBzcGVuZCBvbiBmaXNoaW5nIGFjdGl2aXRpZXMuIFRoaXMgYWxzbyBhZmZlY3RlZCB0aGUgbWFya2V0YWJpbGl0eSBvZiBmaXNoIHByb2R1Y3RzLCByZWR1Y2VkIGluY29tZSBhbmQgaW5jcmVhc2VkIHBvdmVydHkuIFRvIGZ1bGx5IHVuZGVyc3RhbmQgdGhlIGludGVyY29ubmVjdGlvbnMgYmV0d2VlbiB3YXRlciBxdWFsaXR5IGFuZCB2dWxuZXJhYmlsaXR5IGluIFNTRnMgaW4gQ2hpbGlrYSBMYWdvb24sIGl0IGlzIGltcG9ydGFudCB0byBjb25zaWRlciBib3RoIGVudmlyb25tZW50YWwgYW5kIHNvY2lhbCBmYWN0b3JzLCBhcyB3ZWxsIGFzIHRoZSBjb21wbGV4IGZlZWRiYWNrIGxvb3BzIGJldHdlZW4gdGhlc2UgZmFjdG9ycy4gVGhlIHN0dWR5IGhlbHBzIGluIGJyaWRnaW5nIGEgY3J1Y2lhbCBnYXAgaW4gb3VyIHVuZGVyc3RhbmRpbmcgb2YgdGhlIHJvbGUgb2Ygd2F0ZXIgcXVhbGl0eSBpbiB2dWxuZXJhYmlsaXR5IGFuYWx5c2lzIHdpdGhpbiByZXNvdXJjZSBkZXBlbmRlbnQgY29tbXVuaXRpZXMuIFdlIGNvbmNsdWRlIHdpdGgga2V5IGluc2lnaHRzIG9uIHBvc3NpYmxlIGNvcGluZyByZXNwb25zZXMgYW5kIGFkYXB0aXZlIGNhcGFjaXR5IG5lY2Vzc2FyeSBmb3IgdGhlIHNtYWxsLXNjYWxlIGZpc2hlcmllcyBjb21tdW5pdGllcyB0byB0cmFuc2l0aW9uIHRvd2FyZCB2aWFiaWxpdHkuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjMzODkvZm1hcnMuMjAyMy4xMDg3Mjk2IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5mcm9udGllcnNpbi5vcmcvYXJ0aWNsZXMvMTAuMzM4OS9mbWFycy4yMDIzLjEwODcyOTYvcGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjMtMDYtMjkiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTEwIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJGcm9udGllcnMgaW4gTWFyaW5lIFNjaWVuY2UiLCJ2b2x1bWUiOjEwLCJpbXBhY3RGYWN0b3IiOiI0LjI4In0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkZyb250aWVycyBNZWRpYSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NTEsImNpdGVkQnlDb3VudCI6NX19LCJxdW90ZSI6IldhdGVyIHF1YWxpdHkgb2YgQ2hpbGlrYSBMYWdvb24gaGFzIGRlZ3JhZGVkIGR1ZSB0byBhbnRocm9wb2dlbmljIGludGVydmVudGlvbnMgYW5kIG5hdHVyYWwgZm9yY2VzIG9mIGNoYW5nZS4gVmFyaW91cyBhY3Rpdml0aWVzIGluIHRoZSBDaGlsaWthIExhZ29vbiwgc3VjaCBhcyBpbmR1c3RyaWFsIHdhc3Rld2F0ZXIgZGlzcG9zYWwsIHNld2FnZSBkdW1waW5nLCBhcXVhY3VsdHVyZSwgaHlkcm9sb2dpY2FsIGludGVydmVudGlvbnMsIGFuZCBjeWNsb25lcywgYXJlIHByb2R1Y2luZyBzYWxpbml0eSBjaGFuZ2VzLCBzZWRpbWVudCBkZXBvc2l0aW9uLCBudXRyaWVudCBlbnJpY2htZW50LCBldXRyb3BoaWNhdGlvbiwgYW5kIGRlYWQgem9uZXMsIGFzIHN0YXRlZC4gRmlzaCBzdXJ2aXZhbCBpcyBhdCByaXNrIGFzIGEgcmVzdWx0IG9mIHRoZSBzaWduaWZpY2FudCByZWR1Y3Rpb24gaW4gZW52aXJvbm1lbnRhbCBjaXJjdW1zdGFuY2VzLCB3aGljaCBkaW1pbmlzaGVzIGZpc2hlcm1lbuKAmXMgaW5jb21lIGFuZCBwdXNoZXMgdGhlbSBvdXQgb2Ygc29jaWV0eS4gVGhlc2UgY2F1c2VzIGFmZmVjdGluZyB0aGUgd2F0ZXIgcXVhbGl0eSBvZiB0aGUgTGFnb29uIGRlcGljdCBtYW55IGZhY2VzIG9mIHZ1bG5lcmFiaWxpdHkgaW4gU1NGIGNvbW11bml0aWVzLiBBcyB0aGUgQ2hpbGlrYSBMYWdvb24gZW52aXJvbm1lbnQgaXMgZXhwZWN0ZWQgdG8gdW5kZXJnbyBpbmNyZWFzaW5nbHkgZHJhc3RpYyBjaGFuZ2VzIChzdWNoIGFzIGNhdGNoIGNvbXBvc2l0aW9uLCBjYXRjaCBjYXBhY2l0eSwgYmlvZGl2ZXJzaXR5IHZhcmlhdGlvbnMsIHdhdGVyIHF1YWxpdHksIGFuZCBmaXNoZXJ5IHJldmVudWVzKSwgaW50ZXJuYXRpb25hbCBwb2xpY3kgaGFzIGJlY29tZSBtb3JlIGludGVyZXN0ZWQgaW4gYWRhcHRhdGlvbiBtZWFzdXJlcy4gR2l2ZW4gaG93IGRpdmVyc2UgU1NGIGVjb3N5c3RlbXMgYXJlIGV4cGVjdGVkIHRvIGJlIGJldHdlZW4gY29udGluZW50cywgdGhlcmUgaXMgbm8g4oCcb25lIHNpemUgZml0cyBhbGzigJ0gYXBwcm9hY2guIFRoZSBwYXBlciBvdXRsaW5lZCBzZXZlcmFsIGNvcGluZyBhbmQgYWRhcHRhdGlvbiBtZWNoYW5pc21zIHRvIGltcHJvdmUgd2F0ZXIgcXVhbGl0eSBhbmQgY29udGludWUgZmlzaGluZyBpbiBhIHN1c3RhaW5hYmxlIHdheS4gQWRhcHRhdGlvbiBzdHJhdGVnaWVzIGNhbiBvY2Nhc2lvbmFsbHkgaGF2ZSB1bmZvcmVzZWVuIGNvbnNlcXVlbmNlcyBmb3IgZmlzaGVyaWVzIGFuZCBmaXNoaW5nIGNvbW11bml0aWVzLiBBbnkgdW5leHBlY3RlZCBjb25zZXF1ZW5jZXMgb2YgYWRhcHRhdGlvbiBjYW4gYmUgbWl0aWdhdGVkIHRocm91Z2ggbG9uZy10ZXJtIHBsYW5uaW5nIGFuZCBpZGVudGlmaWNhdGlvbiBvZiBmdXR1cmUgb3B0aW9ucyB0aHJvdWdoIHNjZW5hcmlvIGFuYWx5c2lzLiBUaGUgZmluZGluZ3Mgb2YgdGhpcyBwYXBlciBjYW4gYmUgdXRpbGl6ZWQgdG8gZGV2ZWxvcCBwb2xpY2llcyB0byBwcm90ZWN0IHdhdGVyIHF1YWxpdHkgYW5kIFNTRnMgaW4gYSB2YXJpZXR5IG9mIGNvYXN0YWwgbGFnb29uIHNldHRpbmdzLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyMi4yODUwMzYwODcwMzYxMzMsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40OTMwNTM0MzYyNzkzLCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyMy43NzM1MTk1MTU5OTEyMSwibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ4MzM2NDEwNTIyNDYxLCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyNS4yNTU1ODQ3MTY3OTY4NzUsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40ODk1NTkxNzM1ODM5ODQsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjI2Ljc0NDA2ODE0NTc1MTk1MywibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjUwNjgwNTQxOTkyMTg3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjguMjI2MTMzMzQ2NTU3NjE3LCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDkwNDYzMjU2ODM1OTQsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjI5LjcxNDYxNjc3NTUxMjY5NSwibGVmdCI6OC4zMzE1OTI1NTk4MTQ0NTMsIndpZHRoIjo0MC40OTQwOTg2NjMzMzAwOCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MzEuMTk2NjgzODgzNjY2OTkyLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDk0MzkyMzk1MDE5NTMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjMyLjY4NTE2NTQwNTI3MzQ0LCJsZWZ0Ijo4LjMzMTU5NjM3NDUxMTcxOSwid2lkdGgiOjQwLjQ4NzQ0NTgzMTI5ODgzLCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozNC4xNjcyMzI1MTM0Mjc3MzQsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDk3NzQ1NTEzOTE2MDE2LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozNS42NTU3MTU5NDIzODI4MSwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40OTg2MzgxNTMwNzYxNywiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MzcuMTM3NzgzMDUwNTM3MTEsImxlZnQiOjEwLjY4NTE4NjM4NjEwODM5OCwid2lkdGgiOjM4LjE0MTEwNTY1MTg1NTQ3LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozOC42MjYyNjY0Nzk0OTIxOSwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40ODcxNTIwOTk2MDkzNzUsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQwLjEwODMzMzU4NzY0NjQ4NCwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40ODk4NTY3MTk5NzA3LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo0MS41OTY4MTMyMDE5MDQzLCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4ODY2NjUzNDQyMzgzLCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo0My4wNzg4ODAzMTAwNTg1OTQsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDgxODYxMTE0NTAxOTUsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQ0LjU2NzM2MzczOTAxMzY3LCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4MzA1ODkyOTQ0MzM2LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo0Ni4wNDk0MzA4NDcxNjc5NywibGVmdCI6OC4zMzMxMDg5MDE5Nzc1MzksIndpZHRoIjo0MC41MDA3NDc2ODA2NjQwNiwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDcuNTM3OTE0Mjc2MTIzMDUsImxlZnQiOjguMzMzMTA4OTAxOTc3NTM5LCJ3aWR0aCI6NDAuNDk1NjA1NDY4NzUsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQ5LjAxOTk4MTM4NDI3NzM0NCwibGVmdCI6OC4zMzMxMDYwNDA5NTQ1OSwid2lkdGgiOjQwLjUwNjgxNjg2NDAxMzY3LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo1MC41MDg0NjQ4MTMyMzI0MiwibGVmdCI6OC4zMzMxMDYwNDA5NTQ1OSwid2lkdGgiOjQwLjQ4ODUwNjMxNzEzODY3LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo1MS45OTA1MjgxMDY2ODk0NSwibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ5NzcyNjQ0MDQyOTY5LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo1My40NzkwMTE1MzU2NDQ1MywibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjMzLjE1OTQ0NjcxNjMwODU5NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NTQuOTYxMDc4NjQzNzk4ODMsImxlZnQiOjEwLjY4MzY3MDA0Mzk0NTMxMiwid2lkdGgiOjM4LjEzNjU5NjY3OTY4NzUsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjU2LjQ0OTU2MjA3Mjc1MzkwNiwibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ4NzE0NDQ3MDIxNDg0NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIxMyJ9fV0%3D.

These oxygen crises precipitate full ecosystem collapse, eroding biodiversity and resilience. The major ecological consequences of harmful algal blooms are summarized in Table 3. Hypoxic events favor tolerant invasives over native diatoms and macrophytes, homogenizing communities and diminishing ecosystem services like water purification (Shahbaz et al., 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1kIiwiZmFtaWx5IjoiU2hhaGJheiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBaGJhciIsImZhbWlseSI6IkFsYW0ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTW9oYW1tYWQgTWFzcm9vciIsImZhbWlseSI6IlphZmFyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1vaGFtbWVkIEFhc2lmIiwiZmFtaWx5IjoiU3VsYWltYW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW51cG1hIiwiZmFtaWx5IjoiS3VtYXJpIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkcuIEsuIiwiZmFtaWx5IjoiU2hhcm1hIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNoYWhsYSIsImZhbWlseSI6Illhc21pbiJ9fV0sImlkcyI6eyJpc3NuIjpbIjEyMTUtMTYxMCIsIjIwNjEtOTU4OCJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMjQ3OC9vcmh1LTIwMjMtMDAxNiIsIm9wZW5hbGV4IjoiVzQzODk1MTAwMzcifSwidGl0bGUiOiJDb21wYXJpc29uIG9mIGF2aWFuIGRpdmVyc2l0eSBiZXR3ZWVuIG1hbmFnZWQgYW5kIHVubWFuYWdlZCB3ZXRsYW5kcyBpbiBQYXRuYSwgQmloYXIsIEluZGlhIiwiYWJzdHJhY3QiOiJBYnN0cmFjdCBXZSBjb21wYXJlZCB0aGUgYmlyZCBkaXZlcnNpdHkgb2YgYSBtYW5hZ2VkIChSYWpkaGFuaSBKYWxhc2hheSkgYW5kIGFuIHVubWFuYWdlZCB3ZXRsYW5kIChNdXN0YWZhcHVyKSBpbiBQYXRuYSwgQmloYXIsIEluZGlhLiBXZSByZWNvcmRlZCBiaXJkIHNwZWNpZXMgYnkgdXNpbmcgcG9pbnQgY291bnRzIGFuZCBmaXhlZC1yb3V0ZSBtb25pdG9yaW5nLiBBdCBSYWpkaGFuaSBKYWxhc2hheSwgYSB0b3RhbCBvZiA3MyBzcGVjaWVzIHdlcmUgcmVjb3JkZWQsIG9mIHdoaWNoIDE1IHNwZWNpZXMgd2VyZSB3aW50ZXIgdmlzaXRvcnMuIFR3byBzcGVjaWVzIHdlcmUgaW4gdGhlIE5lYXItdGhyZWF0ZW5lZCBjYXRlZ29yeTogRmVycnVnaW5vdXMgRHVjayAoQXl0aHlhIG55cm9jYSkgYW5kIEFsZXhhbmRyaW5lIFBhcmFrZWV0IChQc2l0dGFjdWxhIGV1cGF0cmlhKSAsIHdoaWxlIHRoZSBvdGhlcnMgd2VyZSBvZiBsZWFzdCBjb25jZXJuLiBBdCBNdXN0YWZhcHVyIHdldGxhbmQsIDY3IHNwZWNpZXMgd2VyZSByZWNvcmRlZCB3aXRoIDExIHNwZWNpZXMgYXMgd2ludGVyIHZpc2l0b3JzLiBTaW1wc29u4oCZcyBzcGVjaWVzIGRpdmVyc2l0eSBpbmRleCB3YXMgMC43OCBhdCBSYWpkaGFuaSBKYWxhc2hheSwgYW5kIDAuODEgYXQgTXVzdGFmYXB1ciB3ZXRsYW5kLiBUaGUgbnVtYmVyIG9mIHNwZWNpZXMgb2Ygd2FkZXJzIHdhcyAxNyBhdCBNdXN0YWZhcHVyIHdldGxhbmQgYW5kIDExIGF0IFJhamRoYW5pIEphbGFzaGF5LiBUaGUgdG90YWwgbnVtYmVyIG9mIExlc3NlciBXaGlzdGxpbmctZHVjayAoRGVuZHJvY3lnbmEgamF2YW5pY2EpICwgYSByZXNpZGVudCBiaXJkIHdhcyBhbG1vc3QgZG91YmxlICg1MjYpIGF0IE11c3RhZmFwdXIgd2V0bGFuZCB0aGFuIHRoYXQgYXQgUmFqZGhhbmkgSmFsYXNoYXkgKDIzNCkuIFRoZSBuaXRyYXRlIGxldmVsIHdhcyB0d2ljZSBoaWdoZXIgYW5kIHRoZSBwaG9zcGhhdGUgbGV2ZWwgd2FzIGZvdXIgdGltZXMgaGlnaGVyIGF0IE11c3RhZmFwdXIgd2V0bGFuZCB0aGFuIHRob3NlIGF0IFJhamRoYW5pIEphbGFzaGF5LiBUaGUgZGlmZmVyZW5jZSBpbiBzcGVjaWVzIG51bWJlciBhbmQgY29tcG9zaXRpb24gYmV0d2VlbiB0aGUgdHdvIHdldGxhbmRzIG1heSBiZSBhdHRyaWJ1dGVkIHRvIHRoZSBldXRyb3BoaWNhdGlvbiByZXN1bHRpbmcgZnJvbSBoaWdoIG5pdHJhdGUgYW5kIHBob3NwaGF0ZSBsZXZlbHMgYXQgTXVzdGFmYXB1ciB3ZXRsYW5kLiBVbmZvcnR1bmF0ZWx5LCB0aGUgTXVzdGFmYXB1ciB3ZXRsYW5kIGlzIHVuZGVyIHRocmVhdCBkdWUgdG8gaHVtYW4gaW5mbHVlbmNlcy4gVGhlIHByb3RlY3Rpb24gYW5kIHN1c3RhaW5hYmxlIG1hbmFnZW1lbnQgb2YgbmF0dXJhbCB3ZXRsYW5kcyBpcyByZXF1aXJlZCBmb3Igc2F2aW5nIHRoZSBiaW9kaXZlcnNpdHkgb2YgdGhlIGFyZWEuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjI0Nzgvb3JodS0yMDIzLTAwMTYiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vc2NpZW5kby5jb20vcGRmLzEwLjI0Nzgvb3JodS0yMDIzLTAwMTYiLCJpc3N1ZWREYXRlIjoiMjAyMy0xMi0wMSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6Ik9ybmlzIEh1bmdhcmljYSIsImlzc3VlIjoyLCJ2b2x1bWUiOjMxLCJpbXBhY3RGYWN0b3IiOiIwLjkyIn0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTMsImxhc3RQYWdlIjoyOH19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJIdW5nYXJpYW4gT3JuaXRob2xvZ2ljYWwgYW5kIE5hdHVyZSBDb25zZXJ2YXRpb24gU29jaWV0eSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MjcsImNpdGVkQnlDb3VudCI6MX19LCJxdW90ZSI6Ikhvd2V2ZXIsIGhpZ2ggbGV2ZWxzIG9mIGV1dHJvcGhpY2F0aW9uIG1heSBhbHNvIHRyaWdnZXIgdGhlIHByb2xpZmVyYXRpb24gb2YgZ3JlZW4gYWxnYWUgYW5kICBkZWNsaW5lIG9mIG1hY3JvcGh5dGVzIGxlYWRpbmcgdG8gYSBoaWdoLXR1cmJpZGl0eSBzdGF0ZSBvZiB3YXRlciwgaW4gd2hpY2ggZmV3IHNwZWNpZXMgY2FuICBzdXJ2aXZlIChTY2hlZmZlciBldCBhbC4gMTk5MykuIFRvIGFkZHJlc3MgdGhlIGV1dHJvcGhpY2F0aW9uIGFuZCBleGNlc3MgbnV0cmllbnQgaXNzdWVzIGluICBhcXVhdGljIGVjb3N5c3RlbXMsIHBoeXRvcmVtZWRpYXRpb24gb2ZmZXJzIGEgcHJvbWlzaW5nIHNvbHV0aW9uIChMaXUgZXQgYWwuIDIwMjEpLiBBcXVhdGljICBwbGFudHMgbGlrZSBkdWNrd2VlZHMsIHdhdGVyIGxldHR1Y2UgKFBpc3RpYSBzdHJhdGlvdGVzKSwgYW5kIHdhdGVyIGxpbGllcyBoYXZlIGRlbW9uc3RyYXRlZCAgdGhlaXIgYWJpbGl0eSB0byBjb21iYXQgZXV0cm9waGljYXRpb24gYnkgYWJzb3JiaW5nIGFuZCBhc3NpbWlsYXRpbmcgZXhjZXNzIHBob3NwaG9ydXMgYW5kICBuaXRyb2dlbiAoTmFoYXIgJiBIb3F1ZSAyMDIxKS4gSG93ZXZlciwgdGhlc2UgcGxhbnRzIGNhbiBzb21ldGltZXMgZ3JvdyB1bmNvbnRyb2xsYWJseSwgIHBvdGVudGlhbGx5IGNvdmVyaW5nIHRoZSB3YXRlciBzdXJmYWNlcy4gVG8gYXZvaWQgc3VjaCBzY2VuYXJpb3MsIHJlZ3VsYXIgbW9uaXRvcmluZyBpcyBlc3NlbnRpYWwgKExpdSBldCBhbC4gMjAyMSkuIFByb21vdGluZyBwdWJsaWMgYXdhcmVuZXNzIGlzIHRoZSBjb3JuZXJzdG9uZSBmb3IgbW9iaWxpemluZyBzdXBwb3J0IGFuZCBlbnN1cmluZyBlZmZlY3RpdmUgZXV0cm9waGljYXRpb24gbWFuYWdlbWVudC4gQ29udHJvbGxlZCBlcmFkaWNhdGlvbiBvZiBleGNlc3MgIGZsb3JhIHdoZW4gbmVjZXNzYXJ5IGlzIHZpdGFsIHRvIHByZXZlbnQgb3Zlcmdyb3d0aC4gVGhpcyBpbnRlZ3JhdGVkIGFwcHJvYWNoIGNhbiBlZmZlY3RpdmVseSAgcmVkdWNlIGV1dHJvcGhpY2F0aW9uIGluIHRoZSB3YXRlciBib2RpZXMgKFdhbmcgZXQgYWwuIDIwMjIpLiBFYWNoIHNwZWNpZXMgb2Ygd2V0bGFuZC1kZXBlbmRlbnQgYmlyZCBoYXMgYSB1bmlxdWUgYW5kIGNvbXBsZXggc2V0IG9mIG5lZWRzIGZvciB3ZXRsYW5kIGhhYml0YXRzIHRoYXQgbWFrZXMgaXQgZGlmZmljdWx0IHRvIGdlbmVyYWxpemUgYWJvdXQgaG93IGxvc3Mgb3IgZGVncmFkYXRpb24gb2Ygd2V0bGFuZHMgYWZmZWN0cyBiaXJkIHBvcHVsYXRpb25zLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo3OS4yOTU2MzkwMzgwODU5NCwibGVmdCI6MTAuMzAzMDA2MTcyMTgwMTc2LCJ3aWR0aCI6NzkuOTUyNzI4MjcxNDg0MzgsImhlaWdodCI6MS43MDkxOTc4Nzg4Mzc1ODU0fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjgxLjIxMjUzOTY3Mjg1MTU2LCJsZWZ0IjoxMC4zMDMwMDYxNzIxODAxNzYsIndpZHRoIjo3OS45NTkyODE5MjEzODY3MiwiaGVpZ2h0IjoxLjcwOTE5Nzg3ODgzNzU4NTR9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6ODMuMTI5NDQ3OTM3MDExNzIsImxlZnQiOjEwLjMwMzAwNjE3MjE4MDE3Niwid2lkdGgiOjc5Ljk0MjcyNjEzNTI1MzksImhlaWdodCI6MS43MDkxOTc4Nzg4Mzc1ODU0fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjg1LjA0NjM0ODU3MTc3NzM0LCJsZWZ0IjoxMC4zMDMwMDgwNzk1Mjg4MDksIndpZHRoIjo3OS45Mzk0NTMxMjUsImhlaWdodCI6MS43MDkxOTc4Nzg4Mzc1ODU0fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjg2Ljk2MzI1NjgzNTkzNzUsImxlZnQiOjEwLjMwMzAxMDk0MDU1MTc1OCwid2lkdGgiOjc5Ljk0MTg0ODc1NDg4MjgxLCJoZWlnaHQiOjEuNzA5MTk3ODc4ODM3NTg1NH19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo4OC44ODAxNjUxMDAwOTc2NiwibGVmdCI6MTAuMzAzMDEzODAxNTc0NzA3LCJ3aWR0aCI6NzkuOTM3NSwiaGVpZ2h0IjoxLjcwOTE5Nzg3ODgzNzU4NTR9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6OTAuNzk3MDY1NzM0ODYzMjgsImxlZnQiOjEwLjMwMzAxMzgwMTU3NDcwNywid2lkdGgiOjc5LjkzOTY1MTQ4OTI1NzgxLCJoZWlnaHQiOjEuNzA5MTk3ODc4ODM3NTg1NH19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo5Mi43MTM5NzM5OTkwMjM0NCwibGVmdCI6MTAuMzAzMDEzODAxNTc0NzA3LCJ3aWR0aCI6NzkuOTUwMzI1MDEyMjA3MDMsImhlaWdodCI6MS43MDkxOTc4Nzg4Mzc1ODU0fX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjEyLjIwNzk5MDY0NjM2MjMwNSwibGVmdCI6MTAuMzAzMDI0MjkxOTkyMTg4LCJ3aWR0aCI6NzkuOTUyNTE0NjQ4NDM3NSwiaGVpZ2h0Ijo3LjQ1OTkyMjc5MDUyNzM0NH19LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjoxOS44NzU2MjU2MTAzNTE1NjIsImxlZnQiOjEyLjEyMTMyNDUzOTE4NDU3LCJ3aWR0aCI6NzguMTE5MjI0NTQ4MzM5ODQsImhlaWdodCI6MS43MDkxOTc4Nzg4Mzc1ODU0fX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjIxLjc5MjUzMzg3NDUxMTcyLCJsZWZ0IjoxMC4zMDMwMjkwNjAzNjM3Nywid2lkdGgiOjc5Ljk0OTI0OTI2NzU3ODEyLCJoZWlnaHQiOjEuNzA5MTk3ODc4ODM3NTg1NH19LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjoyMy43MDk0MzY0MTY2MjU5NzcsImxlZnQiOjEwLjMwMzAyOTA2MDM2Mzc3LCJ3aWR0aCI6NzkuOTUyMjcwNTA3ODEyNSwiaGVpZ2h0IjoxLjcwOTE5Nzg3ODgzNzU4NTR9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIyNSJ9fV0%3D. In peri-urban lakes, concretized catchments prolong retention times, sustaining low-DO regimes year-round and fostering perennial dead zones(Paerl, 2018)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhhbnMgVy4iLCJmYW1pbHkiOiJQYWVybCJ9fV0sImlkcyI6eyJpc3NuIjpbXSwiaXNibiI6W10sImRvaSI6IjEwLjE3NjE1L3luNTUtYjQzOCIsIm9wZW5hbGV4IjoiVzQzMDcwNjM3MzkifSwidGl0bGUiOiJNaXRpZ2F0aW5nIHRveGljIHBsYW5rdG9uaWMgY3lhbm9iYWN0ZXJpYWwgYmxvb21zIGluIGFxdWF0aWMgZWNvc3lzdGVtcyBmYWNpbmcgaW5jcmVhc2luZyBhbnRocm9wb2dlbmljIGFuZCBjbGltYXRpYyBwcmVzc3VyZXMiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTc2MTUveW41NS1iNDM4IiwicmVzb3VyY2VVcmwiOiJodHRwczovL2Nkci5saWIudW5jLmVkdS9kb3dubG9hZHMvd2gyNDcyOTBnIiwiaXNzdWVkRGF0ZSI6IjIwMTgtMDEtMDEiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA2IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJDYXJvbGluYSBEaWdpdGFsIFJlcG9zaXRvcnkgKFVuaXZlcnNpdHkgb2YgTm9ydGggQ2Fyb2xpbmEgYXQgQ2hhcGVsIEhpbGwpIiwiaW1wYWN0RmFjdG9yIjoiMC4xMCJ9LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJVbml2ZXJzaXR5IG9mIE5vcnRoIENhcm9saW5hIGF0IENoYXBlbCBIaWxsIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjowLCJjaXRlZEJ5Q291bnQiOjB9fSwicXVvdGUiOiJUaGlzIGxlYWRzIHRvIGNvbnRpbnVpbmcgZGVtYW5kIGZvciBOIHN1cHBsaWVzIHRvIHN1c3RhaW4gcHJpbWFyeSBwcm9kdWN0aW9uIGFuZCBwcm9tb3RlIGV1dHJvcGhpY2F0aW9uIFszOCwzOV0sICgyKSBhIGxlbmd0aHkgbGVnYWN5IG9mIFAgbG9hZGluZyB0aGF0IGlzIHN0b3JlZCBpbiBzZWRpbWVudHMsIHdoaWNoIGlzIHJlYWRpbHkgcmVjeWNsZWQgYW5kIHN1cHBvcnRzIGV1dHJvcGhpY2F0aW9uLCAoMykgTjIgZml4YXRpb24sIHdoaWxlIHByb3ZpZGluZyDigJxuZXfigJ0gTiwgZmFpbHMgdG8gbWVldCBlY29zeXN0ZW0tc2NhbGUgTiByZXF1aXJlbWVudHMgWzQwLDQxXS4gRnJvbSBhIG51dHJpZW50IG1hbmFnZW1lbnQgcGVyc3BlY3RpdmUsIHRoaXMgbWVhbnMgdGhhdCBjb250aW51ZWQgZXh0ZXJuYWwgbG9hZGluZyBvZiBOIHBsYXlzIGEgY3JpdGljYWwgcm9sZSBpbiBhY2NlbGVyYXRpbmcgZXV0cm9waGljYXRpb24gYW5kIHByb21vdGluZyBhbGdhbCBibG9vbXMgWzM2XS4gT25lIHRyb3VibGluZyBpbmRpY2F0b3Igb2YgTi1kcml2ZW4gZXV0cm9waGljYXRpb24gYW5kIEN5YW5vSEFCcyBpbiBmcmVzaHdhdGVyIHN5c3RlbXMgaXMgdGhlIGdlb2dyYXBoaWMgZXhwYW5zaW9uIGFuZCBpbmNyZWFzZSBmcmVxdWVuY3kgb2YgdGhlIG5vbi1OMiBmaXhpbmcgdG94aW4tcHJvZHVjaW5nIGdlbnVzIE1pY3JvY3lzdGlzIFs0Ml0sIGVzcGVjaWFsbHkgaW4gbGFyZ2UgbGFrZXMgKGUuZy4sIExha2UgRXJpZSwgQ2FuYWRhLVVTQSwgTGFrZSBLYXN1bWFndXJhLCBKYXBhbiwgTGFrZSBPa2VlY2hvYmVlLCBGbG9yaWRhLCBVU0EsIExha2UgVGFpaHUsIENoaW5hKS4gVGhpcyBnZW51cywgYXMgd2VsbCBhcyBhbm90aGVyIHBsYW5rdG9uaWMgbm9uLU4yIGZpeGluZyBDeWFub0hBQiBQbGFua3RvdGhyaXgsIGFuZCBzZXZlcmFsIGJlbnRoaWMgYW5hbG9ndWUgYmxvb20gdGF4YSAoZS5nLiwgbm9uLU4yIGZpeGluZyBMeW5nYnlhIGFuZCBPc2NpbGxhdG9yaWEgc3BwLiksIHNlcnZlIGFzIGluZGljYXRvcnMgb2YgTi1vdmVyLWVucmljaG1lbnQgWzM2LDQyXS4gVGhlIHByb2xpZmVyYXRpb24gb2YgdGhlc2Ugbm9uLU4yIGZpeGluZyBDeWFub0hBQnMgaXMgYW4gYWRkaXRpb25hbCBpbmRpY2F0b3IgdGhlIE4gaW5wdXQgcmVkdWN0aW9ucyBhcmUgbmVlZGVkIHRvIHJldmVyc2UgdGhpcyB0cmVuZC4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo3Ny40NDYyNTA5MTU1MjczNCwibGVmdCI6MTYuNDI4NTE0NDgwNTkwODIsIndpZHRoIjo3MC43MTQ3MjE2Nzk2ODc1LCJoZWlnaHQiOjEuNDUwNzk2NDg0OTQ3MjA0Nn19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjc5LjA1MDk3OTYxNDI1NzgxLCJsZWZ0IjoxMi44NTcwNjEzODYxMDgzOTgsIndpZHRoIjo3NC40OTQ5ODc0ODc3OTI5NywiaGVpZ2h0IjoxLjQ1MDc5NjQ4NDk0NzIwNDZ9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo4MC42NTU1ODYyNDI2NzU3OCwibGVmdCI6MTIuODAxOTYxODk4ODAzNzExLCJ3aWR0aCI6NzQuMzQwMDE5MjI2MDc0MjIsImhlaWdodCI6MS40NTA3OTY0ODQ5NDcyMDQ2fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6ODIuMjYwMTg1MjQxNjk5MjIsImxlZnQiOjEyLjg1NzA2MTM4NjEwODM5OCwid2lkdGgiOjc0LjI4NTE4Njc2NzU3ODEyLCJoZWlnaHQiOjEuNjY2OTc3NjQzOTY2Njc0OH19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjgzLjg2NDkwNjMxMTAzNTE2LCJsZWZ0IjoxMi44NTcwNjEzODYxMDgzOTgsIndpZHRoIjo3NC4yODUwMTg5MjA4OTg0NCwiaGVpZ2h0IjoxLjQ1MDc5NjQ4NDk0NzIwNDZ9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo4NS40Njk1MTI5Mzk0NTMxMiwibGVmdCI6MTIuODU3MDYxMzg2MTA4Mzk4LCJ3aWR0aCI6NzEuMTA0OTg4MDk4MTQ0NTMsImhlaWdodCI6MS40NTA3OTY0ODQ5NDcyMDQ2fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6ODcuMDc0MjM0MDA4Nzg5MDYsImxlZnQiOjE2LjQyODUxNDQ4MDU5MDgyLCJ3aWR0aCI6NzAuNzE0MTcyMzYzMjgxMjUsImhlaWdodCI6MS40NTA3OTY0ODQ5NDcyMDQ2fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6ODguNjc4ODQwNjM3MjA3MDMsImxlZnQiOjEyLjg1NzA2MTM4NjEwODM5OCwid2lkdGgiOjc0LjI4NjAyNjAwMDk3NjU2LCJoZWlnaHQiOjEuNjY2OTc3NjQzOTY2Njc0OH19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjkwLjI4MzQzOTYzNjIzMDQ3LCJsZWZ0IjoxMi44NTcwNjEzODYxMDgzOTgsIndpZHRoIjo3NC4yODU0MTU2NDk0MTQwNiwiaGVpZ2h0IjoxLjQ3NjgzMDQ4MjQ4MjkxMDJ9fSx7InBhZ2VOdW1iZXIiOjYsImJib3giOnsidG9wIjoxMC41MDY1MTM1OTU1ODEwNTUsImxlZnQiOjEyLjg1NzA2MTM4NjEwODM5OCwid2lkdGgiOjc0LjIwMjAxODczNzc5Mjk3LCJoZWlnaHQiOjEuNTg2NjA4NDA5ODgxNTkxOH19LHsicGFnZU51bWJlciI6NiwiYmJveCI6eyJ0b3AiOjEyLjExMTIzNzUyNTkzOTk0MSwibGVmdCI6MTIuODU3MDYxMzg2MTA4Mzk4LCJ3aWR0aCI6NzQuMjg1Njc1MDQ4ODI4MTIsImhlaWdodCI6MS42NjY5ODEzMzk0NTQ2NTA5fX0seyJwYWdlTnVtYmVyIjo2LCJiYm94Ijp7InRvcCI6MTMuNzE1ODM3NDc4NjM3Njk1LCJsZWZ0IjoxMi44NTcwNjEzODYxMDgzOTgsIndpZHRoIjo3NC4yODU5NzI1OTUyMTQ4NCwiaGVpZ2h0IjoxLjQ3NjgzNDE3Nzk3MDg4NjJ9fSx7InBhZ2VOdW1iZXIiOjYsImJib3giOnsidG9wIjoxNS4zMjA1NjE0MDg5OTY1ODIsImxlZnQiOjEyLjg1NzA2MTM4NjEwODM5OCwid2lkdGgiOjc0LjI4NTEwMjg0NDIzODI4LCJoZWlnaHQiOjEuNjY2ODY1MzQ4ODE1OTE4fX0seyJwYWdlTnVtYmVyIjo2LCJiYm94Ijp7InRvcCI6MTYuOTI1MTYzMjY5MDQyOTcsImxlZnQiOjEyLjg1NzA2MTM4NjEwODM5OCwid2lkdGgiOjc0LjI4NTM1NDYxNDI1NzgxLCJoZWlnaHQiOjEuNjY2OTgxMzM5NDU0NjUwOX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjYifX1d. Projections under RCP 4.5 warn of 70-80% of Indian lakes becoming chronically hypoxic by 2050, absent interventions(Nõges et al., 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRpaW5hIiwiZmFtaWx5IjoiTsO1Z2VzIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hc2ltZSIsImZhbWlseSI6IkphbmF0aWFuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlJlZXQiLCJmYW1pbHkiOiJMYXVnYXN0ZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQZWV0ZXIiLCJmYW1pbHkiOiJOw7VnZXMifX1dLCJpZHMiOnsiaXNzbiI6WyIyMzUxLTk4OTQiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMTYvai5nZWNjby4yMDIwLmUwMTM2OSIsIm1hZyI6IjMxMDU0NDYwOTAiLCJvcGVuYWxleCI6IlczMTA1NDQ2MDkwIiwiZXhhIjoiaHR0cHM6Ly93d3cuc2NpZW5jZWRpcmVjdC5jb20vc2NpZW5jZS9hcnRpY2xlL3BpaS9TMjM1MTk4OTQyMDMwOTEwMCJ9LCJ0aXRsZSI6IlBvc3Qtc292aWV0IGNoYW5nZXMgaW4gbml0cm9nZW4gYW5kIHBob3NwaG9ydXMgc3RvaWNoaW9tZXRyeSBpbiB0d28gbGFyZ2Ugbm9uLXN0cmF0aWZpZWQgbGFrZXMgYW5kIHRoZSBpbXBhY3Qgb24gcGh5dG9wbGFua3RvbiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2ouZ2VjY28uMjAyMC5lMDEzNjkiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2ouZ2VjY28uMjAyMC5lMDEzNjkiLCJpc3N1ZWREYXRlIjoiMjAyMC0xMS0yMCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTEiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6Ikdsb2JhbCBFY29sb2d5IGFuZCBDb25zZXJ2YXRpb24iLCJ2b2x1bWUiOjI0LCJpbXBhY3RGYWN0b3IiOiIzLjU5In0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkVsc2V2aWVyIEJWIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo3MSwiY2l0ZWRCeUNvdW50IjoxNH19LCJxdW90ZSI6IlJhbmRvbSBGb3Jlc3QgYW5hbHlzaXMgaW5kaWNhdGVkIGFuIGltcG9ydGFudCByb2xlIG9mIGxpZ2h0IGxpbWl0YXRpb24gaW4gYm90aCBsYWtlcy4gQ29uc3RhbnRseSBsb3dlciBsZXZlbHMgb2YgTiBpbiB0aGUgZGVlcGVyIExha2UgUGVpcHNpIGZhdm91cmVkIE4yLWZpeGluZyBzcGVjaWVzLCB3aGljaCwgYXMgYSBwYXJhZG94LCBiZWNhbWUgUC1saW1pdGVkLiBBcyBjbGltYXRlIHdhcm1pbmcgcmVpbmZvcmNlcyBldXRyb3BoaWNhdGlvbiBwaGVub21lbmEgaW4gbGFrZXMgYnkgaW5jcmVhc2luZyBpbnRlcm5hbCBudXRyaWVudCBsb2FkaW5nIGFuZCBmYXZvdXJpbmcgYmxvb20tZm9ybWluZyBjeWFub2JhY3RlcmlhLCBtb3JlIHN0cmluZ2VudCBtZWFzdXJlcyB3b3VsZCBiZSBuZWVkZWQgdG8gZnVydGhlciBsaW1pdCBudXRyaWVudCBsb2FkcyAoZXNwZWNpYWxseSB0aGF0IG9mIFApIHRvIGxha2VzIHRocm91Z2ggaW1wcm92ZWQgd2FzdGV3YXRlciB0cmVhdG1lbnQgYW5kIGluY3JlYXNlZCBlZmZpY2llbmN5IG9mIGZlcnRpbGlzZXIgYXBwbGljYXRpb24uIExvbmctdGVybSBjaGFuZ2VzIGluIG51dHJpZW50IGxvYWRpbmcgYW5kIHN0b2ljaGlvbWV0cnkgaW4gd2F0ZXIgYm9kaWVzIHJlZmxlY3QgY2hhbmdlcyBpbiBhZ3JpY3VsdHVyYWwgcHJhY3RpY2VzIGFuZCB3YXN0ZXdhdGVyIHRyZWF0bWVudC4ifV0%3D. Feedbacks cyanobacterial surface warming, GHG emissions, and nutrient recycling entrench this trajectory, demanding integrated mitigation like aeration and riparian restoration to avert irreversible aquatic deserts (Meerhoff et al., 2022; Paerl et al., 2019)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik0uIiwiZmFtaWx5IjoiTWVlcmhvZmYifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSi4iLCJmYW1pbHkiOiJBdWRldCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJULkEuIiwiZmFtaWx5IjoiRGF2aWRzb24ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTC4gRGUiLCJmYW1pbHkiOiJNZWVzdGVyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlMuIiwiZmFtaWx5IjoiSGlsdCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTLiIsImZhbWlseSI6Iktvc3RlbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJaYWlodWEiLCJmYW1pbHkiOiJMaXUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTi4iLCJmYW1pbHkiOiJNYXp6ZW8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSGFucyBXLiIsImZhbWlseSI6IlBhZXJsIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik0uIiwiZmFtaWx5IjoiU2NoZWZmZXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiRS4iLCJmYW1pbHkiOiJKZXBwZXNlbiJ9fV0sImlkcyI6eyJpc3NuIjpbXSwiaXNibiI6W10sImRvaSI6IjEwLjE3NjE1Lzd2ZTUtdzAxNiIsIm9wZW5hbGV4IjoiVzQzMTg0MTU1NzcifSwidGl0bGUiOiJGZWVkYmFjayBiZXR3ZWVuIGNsaW1hdGUgY2hhbmdlIGFuZCBldXRyb3BoaWNhdGlvbjogcmV2aXNpdGluZyB0aGUgYWxsaWVkIGF0dGFjayBjb25jZXB0IGFuZCBob3cgdG8gc3RyaWtlIGJhY2siLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTc2MTUvN3ZlNS13MDE2IiwicmVzb3VyY2VVcmwiOiJodHRwczovL2Nkci5saWIudW5jLmVkdS9kb3dubG9hZHMvaDk4OXJjOTM4IiwiaXNzdWVkRGF0ZSI6IjIwMjItMDEtMDEiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTEwIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJDYXJvbGluYSBEaWdpdGFsIFJlcG9zaXRvcnkgKFVuaXZlcnNpdHkgb2YgTm9ydGggQ2Fyb2xpbmEgYXQgQ2hhcGVsIEhpbGwpIiwiaW1wYWN0RmFjdG9yIjoiMC4xMCJ9LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJVbml2ZXJzaXR5IG9mIE5vcnRoIENhcm9saW5hIGF0IENoYXBlbCBIaWxsIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjowLCJjaXRlZEJ5Q291bnQiOjN9fSwicXVvdGUiOiJXaXRoIGNsaW1hdGUgd2FybWluZywgdGhlIGRpcmVjdCBhbmQgaW5kaS1yZWN0IGVmZmVjdHMgb2YgaW5jcmVhc2VkIGxha2UgdGVtcGVyYXR1cmVzIHdpbGwgbGlrZWx5IGRyaXZlIGNoYW5nZXMgaW4gY3lhbm9iYWN0ZXJpYWwgdGF4YSBhbmQgdGhlaXIgcHJvZHVjLXRpb24gb2YgY3lhbm9iYWN0ZXJpYWwgdG94aW5zLCBsaWtlbHkgaW5jbHVkaW5nIHByb21vdC1pbmcgdGhlIHNlbGVjdGlvbiBvZiBhIGZldyBNaWNyb2N5c3RpcyBzcGVjaWVzIG9yIHN0cmFpbnMgcHJvZHVjaW5nIGhpZ2hseSB0b3hpYyBtaWNyb2N5c3Rpbi1MUiAoTWFudHpvdWtpIGV0IGFsLiAyMDE4KS4gRGVzcGl0ZSB2YXJpYXRpb25zIGFtb25nIGxha2VzIGluIGEgd2F0ZXItc2NhcGUsIGEgcmVnaW9uYWwgaW5jcmVhc2UgaW4gbWljcm9jeXN0aW4gbWF4aW1hIGFuZCBkdXJhdGlvbiBvZiBibG9vbXMgaGFzIGFsc28gYmVlbiBjbGVhcmx5IGV4cGxhaW5lZCBieSBpbmNyZWFzaW5nIHRlbXBlcmF0dXJlIGluIGNlbnRyYWwgTm9ydGggQW1lcmljYSAoSGF5ZXMgZXQgYWwuIDIwMjApLCB3aXRoIGluY3JlYXNpbmcgcmlza3MgdG8gaHVtYW4gaGVhbHRoLiBHcm93aW5nIGV2aWRlbmNlIGluZGljYXRlcyB0aGF0IGN5YW5vYmFjdGVyaWFsIGRvbWluYW5jZSB3aWxsIGV4ZXJ0IGEgcG9zaXRpdmUgZmVlZGJhY2sgd2l0aCBjbGltYXRlIHdhcm1pbmcgKEhhdmVucyBldCBhbC4gMjAxNiwgUGFlcmwgZXQgYWwuIDIwMjApLiBNYW55IGN5YW5vYmFjdGVyaWFsIHNwZWNpZXMgcHJvbGlmZXJhdGUgb24gdGhlIGxha2Ugc3VyZmFjZSwgYWJzb3JiaW5nIGxpZ2h0LCBlbmhhbmNpbmcgbG9jYWxpemVkIHRlbXBlcmF0dXJlIGluY3JlYXNlcywgYW5kIGluY3JlYXNpbmcgdGhlcm1hbCBzdHJhdGlmaWNhdGlvbiBwb3RlbnRpYWwgKEthaHJ1IGV0IGFsLiAxOTkzKS4gQ3lhbm9iYWN0ZXJpYSBwcm9tb3RlIHRoZSByZWxlYXNlIG9mIEdIR3MsIHBhcnRpY3VsYXJseSBDSDQgKFlhbiBldCBhbC4gMjAxNyksIGR1cmluZyB0aGUgZGVjb21wb3NpdGlvbiBmb2xsb3dpbmcgdGhlIGNvbGxhcHNlIG9mIGEgYmxvb20uIEN5YW5vYmFjdGVyaWEgYWxzbyBwcm9tb3RlIHJlbWluZXJhbGl6YXRpb24gb2YgbnV0cmktZW50cyBkdXJpbmcgYmxvb21zLCBsZWFkaW5nIHRvIG11bHRpcGxlIGJsb29tcyBpbiBhIHNpbmdsZSBzZWFzb24uIFVuZGVyIGV1dHJvcGhpYyBhbmQgd2FybWluZyBjb25kaS10aW9ucywgY3lhbm9iYWN0ZXJpYSBkb21pbmFuY2UgbWF5IHRodXMgYmUgYSBzZWxmLXBlcnBldHVhdGluZyBwcm9waGVjeSAoRmlnLiAyIGFuZCAzKS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo2Ny4wNjM0MDAyNjg1NTQ2OSwibGVmdCI6NTEuNDc4ODAxNzI3Mjk0OTIsIndpZHRoIjozOC42NzM0MjM3NjcwODk4NDQsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6NjguNjk4NTQ3MzYzMjgxMjUsImxlZnQiOjUxLjQ3ODgwMTcyNzI5NDkyLCJ3aWR0aCI6MzguNjc5NDk2NzY1MTM2NzIsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6NzAuMzMzNjg2ODI4NjEzMjgsImxlZnQiOjUxLjQ3ODgwMTcyNzI5NDkyLCJ3aWR0aCI6MzguNjc3NTgxNzg3MTA5Mzc1LCJoZWlnaHQiOjEuNDcxMTAxNTIyNDQ1Njc4N319LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjcxLjk2ODgyNjI5Mzk0NTMxLCJsZWZ0Ijo1MS40Nzg4MDE3MjcyOTQ5Miwid2lkdGgiOjM4LjY3OTEwMzg1MTMxODM2LCJoZWlnaHQiOjEuNDcxMTAxNTIyNDQ1Njc4N319LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjczLjYxMDU3MjgxNDk0MTQsImxlZnQiOjUxLjQ3ODgwMTcyNzI5NDkyLCJ3aWR0aCI6MzguNjc0NDI3MDMyNDcwNywiaGVpZ2h0IjoxLjQ3Mzc0Njg5NTc5MDF9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo3NS4yNDcwMzk3OTQ5MjE4OCwibGVmdCI6NTEuNDc4ODA5MzU2Njg5NDUsIndpZHRoIjozOC42NzQyNjMwMDA0ODgyOCwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo3Ni44ODIxODY4ODk2NDg0NCwibGVmdCI6NTEuNDc4ODA5MzU2Njg5NDUsIndpZHRoIjozOC42ODExNzE0MTcyMzYzMywiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo3OC41MjUyNTMyOTU4OTg0NCwibGVmdCI6NTEuNDc4ODA5MzU2Njg5NDUsIndpZHRoIjozOC42NzU2NjI5OTQzODQ3NjYsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODAuMTYwNDAwMzkwNjI1LCJsZWZ0Ijo1MS40Nzg4MDkzNTY2ODk0NSwid2lkdGgiOjM4LjY4MjAyOTcyNDEyMTA5NCwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo4MS43OTU1Mzk4NTU5NTcwMywibGVmdCI6NTEuNDc4ODA5MzU2Njg5NDUsIndpZHRoIjozOC42NzQ5ODM5NzgyNzE0ODQsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODMuNDM4NjEzODkxNjAxNTYsImxlZnQiOjUxLjQ3ODgwOTM1NjY4OTQ1LCJ3aWR0aCI6MzguNjc2MTg1NjA3OTEwMTU2LCJoZWlnaHQiOjEuNDg2OTczNTI0MDkzNjI4fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODUuMDg5NjMwMTI2OTUzMTIsImxlZnQiOjUxLjQ3ODgwOTM1NjY4OTQ1LCJ3aWR0aCI6NC42ODI4MDUwNjEzNDAzMzIsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODYuNzI0Nzc3MjIxNjc5NjksImxlZnQiOjUzLjQ0MTE3NzM2ODE2NDA2LCJ3aWR0aCI6Ni4xNDAzNTU1ODcwMDU2MTUsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODYuNzI0ODkxNjYyNTk3NjYsImxlZnQiOjYwLjk5NDg3Njg2MTU3MjI2Niwid2lkdGgiOjYuMDM2OTkyNTQ5ODk2MjQsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODYuNzI0ODkxNjYyNTk3NjYsImxlZnQiOjY4LjQ0NjUyNTU3MzczMDQ3LCJ3aWR0aCI6Ni4xNTA3MDU4MTQzNjE1NzIsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODYuNzI0ODkxNjYyNTk3NjYsImxlZnQiOjc2LjAwOTgyNjY2MDE1NjI1LCJ3aWR0aCI6Mi43MjczMzMzMDcyNjYyMzU0LCJoZWlnaHQiOjEuNDcxMTAxNTIyNDQ1Njc4N319LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjg2LjcyNDg5MTY2MjU5NzY2LCJsZWZ0Ijo4MC4xNDk1ODE5MDkxNzk2OSwid2lkdGgiOjEwLjAwODIzNDk3NzcyMjE2OCwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo4OC4zNjAwMzg3NTczMjQyMiwibGVmdCI6NTEuNDc3NjU3MzE4MTE1MjM0LCJ3aWR0aCI6MzguNjcyMDU0MjkwNzcxNDg0LCJoZWlnaHQiOjEuNDcxMTAxNTIyNDQ1Njc4N319LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjkwLjAwMzExMjc5Mjk2ODc1LCJsZWZ0Ijo1MS40Nzc2NTczMTgxMTUyMzQsIndpZHRoIjozOC42NzY0OTg0MTMwODU5NCwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo5MS42MzgyNTIyNTgzMDA3OCwibGVmdCI6NTEuNDc3NjUzNTAzNDE3OTcsIndpZHRoIjozOC42NzU4MzQ2NTU3NjE3MiwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjksImJib3giOnsidG9wIjo2LjQ2ODkzNTk2NjQ5MTY5OSwibGVmdCI6OS44NDAxMzM2NjY5OTIxODgsIndpZHRoIjozOC42ODYzMjUwNzMyNDIxOSwiaGVpZ2h0IjoxNi4yMTIxMjU3NzgxOTgyNDJ9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI5In19LHsib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhhbnMgVy4iLCJmYW1pbHkiOiJQYWVybCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLYXJsIEUuIiwiZmFtaWx5IjoiSGF2ZW5zIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hdGhhbiBTLiIsImZhbWlseSI6IkhhbGwifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiVGltb3RoeSBHLiIsImZhbWlseSI6Ik90dGVuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1lbmd5dWFuIiwiZmFtaWx5IjoiWmh1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhhaSIsImZhbWlseSI6Ilh1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikd1YW5nd2VpIiwiZmFtaWx5IjoiWmh1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkJvcWlhbmciLCJmYW1pbHkiOiJRaW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIxMzIzLTE2NTAiLCIxNDQ4LTYwNTkiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwNzEvbWYxODM5MiIsIm1hZyI6IjI5NDIzMjE1NDIiLCJvcGVuYWxleCI6IlcyOTQyMzIxNTQyIn0sInRpdGxlIjoiTWl0aWdhdGluZyBhIGdsb2JhbCBleHBhbnNpb24gb2YgdG94aWMgY3lhbm9iYWN0ZXJpYWwgYmxvb21zOiBjb25mb3VuZGluZyBlZmZlY3RzIGFuZCBjaGFsbGVuZ2VzIHBvc2VkIGJ5IGNsaW1hdGUgY2hhbmdlIiwiYWJzdHJhY3QiOiJNYW5hZ2luZyBhbmQgbWl0aWdhdGluZyB0aGUgZ2xvYmFsIGV4cGFuc2lvbiBvZiB0b3hpYyBjeWFub2JhY3RlcmlhbCBoYXJtZnVsIGFsZ2FsIGJsb29tcyAoQ3lhbm9IQUJzKSBpcyBhIG1ham9yIGNoYWxsZW5nZSBmYWNpbmcgcmVzZWFyY2hlcnMgYW5kIHdhdGVyIHJlc291cmNlIG1hbmFnZXJzLiBWYXJpb3VzIGFwcHJvYWNoZXMsIGluY2x1ZGluZyBudXRyaWVudCBsb2FkIHJlZHVjdGlvbiwgYXJ0aWZpY2lhbCBtaXhpbmcgYW5kIGZsdXNoaW5nLCBvbW5pdm9yb3VzIGZpc2ggcmVtb3ZhbCwgYWxnYWVjaWRlIGFwcGxpY2F0aW9ucyBhbmQgc2VkaW1lbnQgZHJlZGdpbmcsIGhhdmUgYmVlbiB1c2VkIHRvIHJlZHVjZSBibG9vbSBvY2N1cnJlbmNlcy4gSG93ZXZlciwgbWFuYWdlcnMgbm93IGZhY2UgdGhlIGFkZGl0aW9uYWwgY2hhbGxlbmdlIG9mIGhhdmluZyB0byBhZGRyZXNzIHRoZSBlZmZlY3RzIG9mIGNsaW1hdGUgY2hhbmdlIG9uIHdhdGVyc2hlZCBoeWRyb2xvZ2ljYWwgYW5kIG51dHJpZW50IGxvYWQgZHluYW1pY3MsIHdhdGVyIHRlbXBlcmF0dXJlLCBtaXhpbmcgcmVnaW1lIGFuZCBpbnRlcm5hbCBudXRyaWVudCBjeWNsaW5nLiBSaXNpbmcgdGVtcGVyYXR1cmVzIGFuZCBpbmNyZWFzaW5nIGZyZXF1ZW5jaWVzIGFuZCBtYWduaXR1ZGVzIG9mIGV4dHJlbWUgd2VhdGhlciBldmVudHMsIGluY2x1ZGluZyB0cm9waWNhbCBjeWNsb25lcywgZXh0cmF0cm9waWNhbCBzdG9ybXMsIGZsb29kcyBhbmQgZHJvdWdodHMsIGFsbCBwcm9tb3RlIEN5YW5vSEFCcyBhbmQgYWZmZWN0IHRoZSBlZmZpY2FjeSBvZiBlY29zeXN0ZW0gcmVtZWRpYXRpb24gbWVhc3VyZXMuIFRoZXNlIGNsaW1hdGljIGNoYW5nZXMgd2lsbCBsaWtlbHkgcmVxdWlyZSBzZXR0aW5nIHN0cmljdGVyIG51dHJpZW50IChpbmNsdWRpbmcgYm90aCBuaXRyb2dlbiBhbmQgcGhvc3Bob3J1cykgcmVkdWN0aW9uIHRhcmdldHMgZm9yIGJsb29tIGNvbnRyb2wgaW4gYWZmZWN0ZWQgd2F0ZXJzLiBJbiBhZGRpdGlvbiwgdGhlIGVmZmljYWN5IG9mIGN1cnJlbnRseSB1c2VkIG1ldGhvZHMgdG8gcmVkdWNlIEN5YW5vSEFCcyB3aWxsIG5lZWQgdG8gYmUgcmUtZXZhbHVhdGVkIGluIGxpZ2h0IG9mIHRoZSBzeW5lcmdpc3RpYyBlZmZlY3RzIG9mIGNsaW1hdGUgY2hhbmdlIHdpdGggbnV0cmllbnQgZW5yaWNobWVudC4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTA3MS9tZjE4MzkyIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2Nkci5saWIudW5jLmVkdS9kb3dubG9hZHMvMDU3NDIyODM0IiwiaXNzdWVkRGF0ZSI6IjIwMTktMDMtMjUiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTEwIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJNYXJpbmUgYW5kIEZyZXNod2F0ZXIgUmVzZWFyY2giLCJpc3N1ZSI6NSwidm9sdW1lIjo3MSwiaW1wYWN0RmFjdG9yIjoiMi43MCJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjU3OSwibGFzdFBhZ2UiOjU3OX19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJDU0lSTyBQdWJsaXNoaW5nIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjoxMDQsImNpdGVkQnlDb3VudCI6OTF9fSwicXVvdGUiOiJUaGF0IHJlbGF0aW9uc2hpcCBoYXMgc2VydmVkIGFzIHRoZSBmb3VuZGF0aW9uIGZvciBzZXR0aW5nICBudXRyaWVudCBsb2FkaW5nIHRhcmdldHMgaW4gbGFrZXMgYXJvdW5kIHRoZSB3b3JsZCwgYW5kIGNhcmVmdWxseSAgZGVzaWduZWQgZXhwZXJpbWVudHMgbWF5IGVsdWNpZGF0ZSB0byB3aGF0IGV4dGVudCB0ZW1wZXJhdHVyZSAgaW5jcmVhc2UgYWZmZWN0cyB0aGUgdGhyZXNob2xkIGF0IHdoaWNoIHRoZSBmcmVxdWVuY3kgb2Ygb2NjdXIgIHJlbmNlIG9mIEN5YW5vSEFCcyBxdWlja2x5IHJpc2VzIHdpdGggaW5jcmVhc2luZyBOIGFuZCBQLiBDb25jbHVzaW9ucyBSZWNlbnQgc3R1ZGllcyBpbmRpY2F0ZSB0aGF0IHRoZSBzeW5lcmdpc3RpYyBjb21iaW5hdGlvbiBvZiBhbnRocm9wb2dlbmljIG51dHJpZW50IGxvYWRpbmcsIHJpc2luZyB0ZW1wZXJhdHVyZXMsIGVuaGFuY2VkICB2ZXJ0aWNhbCBzdHJhdGlmaWNhdGlvbiwgaW5jcmVhc2VkIHJlc2lkZW5jZSB0aW1lIGFuZCBtb3JlICBleHRyZW1lIGNsaW1hdGljIGNvbmRpdGlvbnMgb3ZlcmFsbCB3aWxsIGZhdm91ciBjeWFub2JhY3RlcmlhbCAgZG9taW5hbmNlIGluIGEgd2lkZSByYW5nZSBvZiBhcXVhdGljIGVjb3N5c3RlbXMuIFRoZSBleHBhbiAgc2lvbiBvZiBDeWFub0hBQnMgaGFzIHNlcmlvdXMgY29uc2VxdWVuY2VzIGZvciBodW1hbiB3YXRlciAgc3VwcGxpZXMsIGZpc2hlcmllcyBhbmQgcmVjcmVhdGlvbmFsIHJlc291cmNlcy4gRWNvc3lzdGVtIHNjYWxlICBwaHlzaWNhbCBjaGVtaWNhbCBiaW90aWMgbWl0aWdhdGlvbiBzdHJhdGVnaWVzIHdpbGwgaGF2ZSB0byAgaW5jb3Jwb3JhdGUgbnV0cmllbnQgKG1vc3Qgb2Z0ZW4gYm90aCBOIGFuZCBQKSBpbnB1dCByZWR1Y3Rpb25zLiAgVGhlIG1hZ25pdHVkZXMgb2YgdGhlc2UgcmVkdWN0aW9ucyB3aWxsIG5lZWQgdG8gYmUgc3lzdGVtICBzcGVjaWZpYyBhbmQgd2lsbCBsaWtlbHkgbmVlZCB0byBiZSBhZGp1c3RlZCB3aXRoIGNoYW5naW5nIGNsaSAgbWF0aWMgY29uZGl0aW9ucy4gU3BlY2lmaWNhbGx5LCB3YXRlciBxdWFsaXR5IG1hbmFnZXJzIHdpbGwgaGF2ZSAgdG8gYWNjb21tb2RhdGUgd2FybWluZyBhbmQgZ3JlYXRlciBoeWRyb2xvZ2ljYWwgdmFyaWFiaWxpdHkgIGFuZCBleHRyZW1lbmVzcyByZXN1bHRpbmcgZnJvbSBjbGltYXRlIGNoYW5nZSBpbiBmb3JtdWxhdGluZyAgdGhlaXIgc3RyYXRlZ2llcy4gQSBrZXkgbG9uZyB0ZXJtIGNvbnRyb2wgd2UgY2FuIGV4ZXJ0IHRvIHJlZHVjZSAgdGhlIHJhdGUgYW5kIGV4dGVudCBvZiBnbG9iYWwgd2FybWluZyBpcyBjdXJiaW5nIGdyZWVuaG91c2UgZ2FzICBlbWlzc2lvbnMuIFdpdGhvdXQgdGhpcyBlc3NlbnRpYWwgc3RlcCwgZnV0dXJlIHdhcm1pbmcgdHJlbmRzLCAgaHlkcm9sb2dpY2FsIGV4dHJlbWVzIChyZWNvcmQgcmFpbmZhbGxzIGFuZCBkcm91Z2h0cykgYW5kIHRoZWlyICBlZmZlY3RzIG9uIGFxdWF0aWMgZWNvc3lzdGVtcyB3aWxsIGNvbnRpbnVlIHRvIHByb21vdGUgb3Bwb3IgIHR1bmlzdGljIEN5YW5vSEFCcy4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6MTQuMTQ5MjMzODE4MDU0MiwibGVmdCI6MTMuMTI4NTk1MzUyMTcyODUyLCJ3aWR0aCI6MzcuMDgyMTcyMzkzNzk4ODMsImhlaWdodCI6MS41MTY2MDIyNzc3NTU3MzczfX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjE1LjQ1MTUwODUyMjAzMzY5MSwibGVmdCI6MTMuMTI4NTQ0ODA3NDM0MDgyLCJ3aWR0aCI6MzcuMDg0ODMxMjM3NzkyOTcsImhlaWdodCI6MS41MTY2MDMyMzE0MzAwNTM3fX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjE2Ljc1Mzc4NDE3OTY4NzUsImxlZnQiOjEzLjEyODU0MjkwMDA4NTQ1LCJ3aWR0aCI6MzcuMDgzMjA5OTkxNDU1MDgsImhlaWdodCI6MS41MTY2MDQxODUxMDQzNzAxfX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjE4LjA1NjA1MzE2MTYyMTA5NCwibGVmdCI6MTMuMTI4NTQ0ODA3NDM0MDgyLCJ3aWR0aCI6MzYuNjE1MDc0MTU3NzE0ODQ0LCJoZWlnaHQiOjEuNTE2NjA0MTg1MTA0MzcwMX19LHsicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjoxOS40NjgzNzgwNjcwMTY2LCJsZWZ0IjoxMy4xMjg1NDQ4MDc0MzQwODIsIndpZHRoIjozNC4yOTA1MzExNTg0NDcyNjYsImhlaWdodCI6MS4yNDUwMTEyMTA0NDE1ODk0fX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjIyLjIyNjE0Mjg4MzMwMDc4LCJsZWZ0IjoxMy4xMjg1NTA1Mjk0Nzk5OCwid2lkdGgiOjcuNjQ5ODczNzMzNTIwNTA4LCJoZWlnaHQiOjEuMTM1NDgzMTQ1NzEzODA2Mn19LHsicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjoyMy43MzQ4NDk5Mjk4MDk1NywibGVmdCI6MTMuMTI4NTM1MjcwNjkwOTE4LCJ3aWR0aCI6MzcuMDg5Mjg2ODA0MTk5MjIsImhlaWdodCI6MjYuMDg4NjQwMjEzMDEyNjk1fX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNTg5In19XQ%3D%3D. When waters enter an anaerobic state, aquatic organisms die in large numbers and the release of sediment-reducing pollutants further deteriorates water quality (Ji et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkRhb2JpbiIsImZhbWlseSI6IkppIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllhbnhpbmciLCJmYW1pbHkiOiJIYW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTGlhbmdob25nIiwiZmFtaWx5IjoiTG9uZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJYaWFva2FuZyIsImZhbWlseSI6IlhpbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJIdWl5YW4iLCJmYW1pbHkiOiJYdSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTaXFpYW4iLCJmYW1pbHkiOiJRaXUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSmlhbmdodWFpIiwiZmFtaWx5IjoiTWVuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJYaW5neGluZyIsImZhbWlseSI6IlpoYW8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiWWFuYW4iLCJmYW1pbHkiOiJIdWFuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJEZWZ1IiwiZmFtaWx5IjoiTGl1In19XSwiaWRzIjp7Imlzc24iOlsiMjI5Ni03MDFYIl0sImlzYm4iOltdLCJkb2kiOiIxMC4zMzg5L2Zldm8uMjAyMy4xMjk3MDQ3Iiwib3BlbmFsZXgiOiJXNDM5MDYxMDY2OCIsImV4YSI6Imh0dHBzOi8vd3d3LmZyb250aWVyc2luLm9yZy9hcnRpY2xlcy8xMC4zMzg5L2Zldm8uMjAyMy4xMjk3MDQ3L2Z1bGwifSwidGl0bGUiOiJIeXBveGlhIGFuZCBpdHMgZmVlZGJhY2sgcmVzcG9uc2UgdG8gYWxnYWwgYmxvb21zIGFuZCBDSDQgZW1pc3Npb25zIGluIHN1YnRyb3BpY2FsIHJlc2Vydm9pcnMiLCJhYnN0cmFjdCI6IkFudGhyb3BvZ2VuaWMgbnV0cmllbnQgaW5wdXQgc3RpbXVsYXRlcyBzdXJmYWNlIGV1dHJvcGhpY2F0aW9uIGFuZCBhbGdhbCBibG9vbXMgYW5kIHRodXMgZXhhY2VyYmF0ZXMgb3h5Z2VuIGRlcGxldGlvbiBhbmQgaHlwb3hpYSBpbiBhcXVhdGljIGVjb3N5c3RlbXMuIEhvd2V2ZXIsIGV4aXN0aW5nIHN0dWRpZXMgaGF2ZSBwYWlkIGxpdHRsZSBhdHRlbnRpb24gb24gdGhlIHVuZGVyc3RhbmRpbmcgb2YgdGhlIGZlZWRiYWNrIHJlbGF0aW9uc2hpcHMgYW1vbmcgb3h5Z2VuIGNvbnN1bXB0aW9uLCBhbGdhbCBzdWNjZXNzaW9uIGFuZCBncmVlbmhvdXNlIGdhcyBlZmZlY3QgYWZ0ZXIgcml2ZXIgZGFtbWluZywgcGFydGljdWxhcmx5IGluIFRocmVlIEdvcmdlcyBSZXNlcnZvaXIuIFRoaXMgZmllbGQgd29yayBpbnZlc3RpZ2F0ZWQgdGhlIHRoZXJtYWwgc3RyYXRpZmljYXRpb24sIHdhdGVyIHF1YWxpdHkgYW5kIHBoeXRvcGxhbmt0b24gaW4gZm91ciB0cmlidXRhcmllcyAoUGVuZ3hpIEJheSAoUFhCKSwgRGFuaW5nIEJheSAoRE5CKSwgU2hlbm5vbmcgQmF5IChTTkIpLCBhbmQgWGlhbmd4aSBCYXkgKFhYQikpIG9mIHRoZSBUaHJlZSBHb3JnZXMgUmVzZXJ2b2lyIChUR1IpIGluIHNwcmluZyAoQXByaWwgMjUtMjgsIDIwMjIpLCBhbmQgZXhwbG9yZWQgdGhlIGRyaXZlcnMgb2YgaHlwb3hpYSBhbmQgaXRzIGZlZWRiYWNrIHJlc3BvbnNlIHRvIGFsZ2FsIGJsb29tcyBhbmQgbWV0aGFuZSAoQ0ggNCApIGVtaXNzaW9ucy4gVGhlIHJlc3VsdHMgc2hvd2VkIHRoYXQgZHVyaW5nIHRoZSBvYnNlcnZhdGlvbiBwZXJpb2QsIERPLCB0aGVybWFsIHN0cmF0aWZpY2F0aW9uLCBhbmQgYWxnYWwgYmxvb21zIHdlcmUgbW9yZSBzZXZlcmUgaW4gUFhCLiBUaGUgaGlnaCBDaGVtaWNhbCBPeHlnZW4gRGVtYW5kIChDT0QgTW4gKSAoMTEuMjcgbWfigKJMIC0xICkgYW5kIERpc3NvbHZlZCBPcmdhbmljIENhcmJvbiAoRE9DKSBjb250ZW50ICg3LjU2IG1n4oCiTCAtMSApIGluIFBYQiB3ZXJlIHRoZSBtYWluIGNhdXNlcyBvZiB0aGUgcHJvZ3Jlc3NpdmUgaHlwb3hpYS4gRnVydGhlcm1vcmUsIHN0cm9uZ2VyIGRlbnNpdHkgc3RyYXRpZmljYXRpb24gaW4gUFhCIGluaGliaXRlZCB0aGUgdmVydGljYWwgc3VwcGx5IG9mIERPIGFuZCBhY2NlbGVyYXRlZCBib3R0b20gaHlwb3hpYS4gQnV0IHRoZSBkb3duc2xvcGUgZGVuc2l0eSBjdXJyZW50IHJlcGxlbmlzaGVkIGJvdHRvbSBETyBpbiB0aGUgdXBwZXIgcmVhY2hlcyBvZiB0aGUgRE5CLCBTTkIsIGFuZCBYWEIuIFRoZSBDSDQgY29uY2VudHJhdGlvbiB3YXMgZ2VuZXJhbGx5IGhpZ2hlciBpbiBoeXBveGlhIHpvbmUsIGVzcGVjaWFsbHkgaW4gd2hlcmUgY3lhbm9iYWN0ZXJpYSBhcmUgdGhlIGRvbWluYW50IHNwZWNpZXMgKGVnLiwgUFgwMyBhbmQgUFgwNCkuIERhbSBjb25zdHJ1Y3Rpb24gbGVkIHRvIHRoZSBzdWNjZXNzaW9uIG9mIGFsZ2FsLWRvbWluYW50IHNwZWNpZXMsIGV4ZXJ0aW5nIGEgc2lnbmlmaWNhbnQgaW5mbHVlbmNlIG9uIHRoZSByaXZlciBjYXJib24gY3ljbGUuIFRoZSBleHBhbnNpb24gb2YgY3lhbm9iYWN0ZXJpYWwgYmxvb21zIGFuZCB0aGUgY29uc2VxdWVuY2Ugb2YgbWV0YWJvbGljIHN1YnN0cmF0ZXMgaW5jcmVhc2UgYXJlIGdyYWR1YWxseSBpbmNyZWFzaW5nIG94eWdlbiBkZXByaXZhdGlvbiwgZXZlbiBoeXBveGlhLCBhbmQgZW5oYW5jaW5nIENIIDQgcHJvZHVjdGlvbiBpbiByaXZlcnMuIFRoZXNlIGZpbmRpbmdzIHN1Z2dlc3QgdGhhdCB0aGUgc3VjY2Vzc2lvbiBvZiBhbGdhbC1kb21pbmFudCBzcGVjaWVzIGNhdXNlZCBieSByaXZlciBkYW1taW5nIGhhcyBhIHByb25vdW5jZWQgcG9zaXRpdmUgZmVlZGJhY2sgZWZmZWN0IG9uIHJlc2Vydm9pciBoeXBveGlhIHByb2Nlc3NlcyBhbmQgZ3JlZW5ob3VzZSBlZmZlY3QuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjMzODkvZmV2by4yMDIzLjEyOTcwNDciLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vd3d3LmZyb250aWVyc2luLm9yZy9hcnRpY2xlcy8xMC4zMzg5L2Zldm8uMjAyMy4xMjk3MDQ3L3BkZj9pc1B1Ymxpc2hlZFYyPUZhbHNlIiwiaXNzdWVkRGF0ZSI6IjIwMjQtMDEtMDUiLCJhY2Nlc3NlZERhdGUiOiIyMDI2LTAyIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJGcm9udGllcnMgaW4gRWNvbG9neSBhbmQgRXZvbHV0aW9uIiwidm9sdW1lIjoxMSwiaW1wYWN0RmFjdG9yIjoiMi42MiJ9LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJGcm9udGllcnMgTWVkaWEifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjQyLCJjaXRlZEJ5Q291bnQiOjh9fSwicXVvdGUiOiJvZ2VuaWMgbnV0cmllbnQgaW5wdXQgc3RpbXVsYXRlcyBzdXJmYWNlIGV1dHJvcGhpY2F0aW9uIGFuZCBhbGdhbCBibG9vbXMgYW5kIHRodXMgZXhhY2VyYmF0ZXMgb3h5Z2VuIGRlcGxldGlvbiBhbmQgaHlwb3hpYSBpbiBhcXVhdGljIGVjb3N5c3RlbXMuIEhvd2V2ZXIsIGV4aXN0aW5nIHN0dWRpZXMgaGF2ZSBwYWlkIGxpdHRsZSBhdHRlbnRpb24gb24gdGhlIHVuZGVyc3RhbmRpbmcgb2YgdGhlIGZlZWRiYWNrIHJlbGF0aW9uc2hpcHMgYW1vbmcgb3h5Z2VuIGNvbnN1bXB0aW9uLCBhbGdhbCBzdWNjZXNzaW9uIGFuZCBncmVlbmhvdXNlIGdhcyBlZmZlY3QgYWZ0ZXIgcml2ZXIgZGFtbWluZywgLi4uIERPIHBsYXlzIGEga2V5IHJvbGUgaW4gdGhlIGJpb2dlb2NoZW1pY2FsIGN5Y2xlIG9mIHN1cmZhY2Ugd2F0ZXJzIGFuZCBpcyBhIHNlbnNpdGl2ZSBpbmRpY2F0b3Igb2YgcGh5c2ljYWwgYW5kIGJpb2dlb2NoZW1pY2FsIGNoYW5nZXMgaW4gcmVzZXJ2b2lyIGFxdWF0aWMgZWNvc3lzdGVtcyAoIFdldHplbCwgMjAwMSApLiBXaGVuIHdhdGVycyBlbnRlciBhbiBhbmFlcm9iaWMgc3RhdGUsIGFxdWF0aWMgb3JnYW5pc21zIGRpZSBpbiBsYXJnZSBudW1iZXJzIGFuZCB0aGUgcmVsZWFzZSBvZiBzZWRpbWVudC1yZWR1Y2luZyBwb2xsdXRhbnRzIChlLmcuLCBhbW1vbmlhIG5pdHJvZ2VuLCAuLi4gMjAyMl0gKSwgZnVydGhlciBkZXRlcmlvcmF0aW5nIHdhdGVyIHF1YWxpdHkuIFdpdGggdGhlIGluY3JlYXNlIG9mIHBvbGx1dGlvbiBsb2FkcyBpbiB3YXRlcnNoZWRzIGFuZCB0aGUgaW50ZW5zaWZpY2F0aW9uIG9mIHdhdGVyIGV1dHJvcGhpY2F0aW9uLCB3YXRlciBoeXBveGlhIGhhcyBiZWNvbWUgYSBzZXZlcmUgZ2xvYmFsIGVjb2xvZ2ljYWwgYW5kIGVudmlyb25tZW50YWwgcHJvYmxlbSBpbiByZXNlcnZvaXJzICggU3VuIGV0wqBhbC4sIDIwMjEgOyBKYWlzd2FsIGV0wqBhbC4sIDIwMjMgKS4gRm9jdXNpbmcgb24gdGhlIGR5bmFtaWMgY2hhbmdlcyBpbiIsImV4dGVybmFsQ2FjaGVJZCI6IjkxMDQ0ODJhLTYwNjYtM2M5YS1hMGE3LTZkZGVmNTRhMTI3ZiJ9XQ%3D%3D. This internal loading mechanism creates a self-reinforcing cycle where anoxic conditions during one summer beget increasingly severe hypoxia in subsequent seasons (Jansen et al., 2024; Lewis et al., 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkpvYWNoaW0iLCJmYW1pbHkiOiJKYW5zZW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiR2F2aW4gTC4iLCJmYW1pbHkiOiJTaW1wc29uIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikdlc2EgQS4iLCJmYW1pbHkiOiJXZXloZW5tZXllciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJMYXVyYSBILiIsImZhbWlseSI6IkjDpHJrw7ZuZW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW5kcmV3IE0uIiwiZmFtaWx5IjoiUGF0ZXJzb24ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUGF1bCBBLiBkZWwiLCJmYW1pbHkiOiJHaW9yZ2lvIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ill2ZXMgVC4iLCJmYW1pbHkiOiJQcmFpcmllIn19XSwiaWRzIjp7Imlzc24iOlsiMTc1OC02NzhYIiwiMTc1OC02Nzk4Il0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDM4L3M0MTU1OC0wMjQtMDIwNTgtMyIsIm9wZW5hbGV4IjoiVzQ0MDAyMDgyMDAiLCJleGEiOiJodHRwczovL3d3dy5uYXR1cmUuY29tL2FydGljbGVzL3M0MTU1OC0wMjQtMDIwNTgtMz9lcnJvcj1jb29raWVzX25vdF9zdXBwb3J0ZWQmY29kZT1hNzRhMjVmZi1hNThkLTRkZjgtOTM2Zi0yNjJlMmUzZTkxOTgifSwidGl0bGUiOiJDbGltYXRlLWRyaXZlbiBkZW94eWdlbmF0aW9uIG9mIG5vcnRoZXJuIGxha2VzIiwiYWJzdHJhY3QiOiJBYnN0cmFjdCBPeHlnZW4gZGVwbGV0aW9uIGNvbnN0aXR1dGVzIGEgbWFqb3IgdGhyZWF0IHRvIGxha2UgZWNvc3lzdGVtcyBhbmQgdGhlIHNlcnZpY2VzIHRoZXkgcHJvdmlkZS4gTW9zdCBvZiB0aGUgd29ybGTigJlzIGxha2VzIGFyZSBsb2NhdGVkICZndDs0NcKwIE4sIHdoZXJlIGFjY2VsZXJhdGVkIGNsaW1hdGUgd2FybWluZyBhbmQgZWxldmF0ZWQgY2FyYm9uIGxvYWRzIG1pZ2h0IHNldmVyZWx5IGluY3JlYXNlIHRoZSByaXNrIG9mIGh5cG94aWEsIGJ1dCB0aGlzIGhhcyBub3QgYmVlbiBzeXN0ZW1hdGljYWxseSBleGFtaW5lZC4gSGVyZSBhbmFseXNpcyBvZiAyLjYgbWlsbGlvbiB3YXRlciBxdWFsaXR5IG9ic2VydmF0aW9ucyBmcm9tIDgsMjg4IGxha2VzIHNob3dzIHRoYXQgYmV0d2VlbiAxOTYwIGFuZCAyMDIyLCBtb3N0IG5vcnRoZXJuIGxha2VzIGV4cGVyaWVuY2VkIHJhcGlkIGRlb3h5Z2VuYXRpb24gc3Ryb25nbHkgbGlua2VkIHRvIGNsaW1hdGUtZHJpdmVuIHByb2xvbmdhdGlvbiBvZiBzdW1tZXIgc3RyYXRpZmljYXRpb24uIE94eWdlbiBsZXZlbHMgZGV0ZXJpb3JhdGVkIG1vc3QgaW4gc21hbGwgbGFrZXMgKCZsdDsxMCBoYSkgb3dpbmcgdG8gdGhlaXIgZ3JlYXRlciB2b2x1bWV0cmljIG94eWdlbiBkZW1hbmQgYW5kIHN1cmZhY2Ugd2FybWluZyByYXRlcywgd2hpbGUgdGhlIGxhcmdlc3QgbGFrZXMgZ2FpbmVkIG94eWdlbiB1bmRlciBtaW5pbWFsIHN0cmF0aWZpY2F0aW9uIGNoYW5nZXMgYW5kIGltcHJvdmVkIGFlcmF0aW9uIGF0IHNwcmluZyBvdmVydHVybnMuIFNlYXNvbmFsIG94eWdlbiBjb25zdW1wdGlvbiByYXRlcyBkZWNsaW5lZCwgZGVzcGl0ZSB3aWRlc3ByZWFkIGJyb3duaW5nLiBQcm9saWZlcmF0aW5nIGFub3hpYSBlbmhhbmNlZCBzZWFzb25hbCBpbnRlcm5hbCBsb2FkaW5nIG9mIEMsIFAgYW5kIE4gYnV0IGRlcGxldGVkIFAgbG9uZy10ZXJtLCBpbmRpY2F0aW5nIHRoYXQgZGVveHlnZW5hdGlvbiBjYW4gZXhoYXVzdCByZWRveC1zZW5zaXRpdmUgZnJhY3Rpb25zIG9mIHNlZGltZW50IG51dHJpZW50IHJlc2Vydm9pcnMuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjEwMzgvczQxNTU4LTAyNC0wMjA1OC0zIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAzOC9zNDE1NTgtMDI0LTAyMDU4LTMiLCJpc3N1ZWREYXRlIjoiMjAyNC0wNy0wMSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTEiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6Ik5hdHVyZSBDbGltYXRlIENoYW5nZSIsImlzc3VlIjo4LCJ2b2x1bWUiOjE0LCJpbXBhY3RGYWN0b3IiOiIxNS4wMiJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjgzMiwibGFzdFBhZ2UiOjgzOH19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJOYXR1cmUgUG9ydGZvbGlvIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo4MCwiY2l0ZWRCeUNvdW50IjozN319LCJxdW90ZSI6IkFlcm9iaWMgcmVzcGlyYXRpb24gY2FuIGRlcGxldGUgb3h5Z2VuIGNvbmNlbnRyYXRpb25zIGF0IGRlcHRoIHdoZW4gc2Vhc29uYWwgc3RyYXRpZmljYXRpb24gcHJldmVudHMgYWVyYXRpb24gb2YgdGhlIHdhdGVyIGNvbHVtbls1XShodHRwczovL3d3dy5uYXR1cmUuY29tL2FydGljbGVzL3M0MTU1OC0wMjQtMDIwNTgtMyNyZWYtQ1I1KS4gVGhpcyBkZXRyaW1lbnRhbCBjb25kaXRpb24sIHRlcm1lZCBoeXBveGlhIChET%2BKAiTzigIky4oCJbWfigIls4oiSMSksIGRpbWluaXNoZXMgdGhlIGNvbGQtd2F0ZXIgaGFiaXRhdCBvZiBhcXVhdGljIGxpZmUsIGluY2x1ZGluZyBjb21tZXJjaWFsbHkgaW1wb3J0YW50IGZpc2hbNl0oaHR0cHM6Ly93d3cubmF0dXJlLmNvbS9hcnRpY2xlcy9zNDE1NTgtMDI0LTAyMDU4LTMjcmVmLUNSNikgYW5kIGNhbiBjYXVzZSBtYXNzIG1vcnRhbGl0eSBpZiBwZXJzaXN0ZW50WzddKGh0dHBzOi8vd3d3Lm5hdHVyZS5jb20vYXJ0aWNsZXMvczQxNTU4LTAyNC0wMjA1OC0zI3JlZi1DUjcpLiBBbm94aWEgKERP4oCJPOKAiTAuNeKAiW1n4oCJbOKIkjEpIHByb21vdGVzIHRoZSByZWR1Y3RpdmUgZGlzc29sdXRpb24gb2YgbWluZXJhbC1ib3VuZCBjYXJib24gYW5kIHBob3NwaG9ydXNbOF0oaHR0cHM6Ly93d3cubmF0dXJlLmNvbS9hcnRpY2xlcy9zNDE1NTgtMDI0LTAyMDU4LTMjcmVmLUNSOCksIFs5XShodHRwczovL3d3dy5uYXR1cmUuY29tL2FydGljbGVzL3M0MTU1OC0wMjQtMDIwNTgtMyNyZWYtQ1I5KSB3aGljaCBmdWVscyBoYXJtZnVsIGFsZ2FsIGJsb29tc1sxMF0oaHR0cHM6Ly93d3cubmF0dXJlLmNvbS9hcnRpY2xlcy9zNDE1NTgtMDI0LTAyMDU4LTMjcmVmLUNSMTApIGFuZCBsaW1pdHMgbWljcm9iaWFsIG94aWRhdGlvbiBvZiBtZXRoYW5lLCBhIHBvdGVudCBncmVlbmhvdXNlIGdhc1sxMV0oaHR0cHM6Ly93d3cubmF0dXJlLmNvbS9hcnRpY2xlcy9zNDE1NTgtMDI0LTAyMDU4LTMjcmVmLUNSMTEpLiIsImV4dGVybmFsQ2FjaGVJZCI6ImMwYWFhMTZiLTA2MDUtM2Y4YS1hYmJiLWE2ZDM5OTQwZDVmYSJ9LHsib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFiaWdhaWwgUy4gTC4iLCJmYW1pbHkiOiJMZXdpcyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNYXhpbWlsaWFuIFAuIiwiZmFtaWx5IjoiTGF1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlN0ZXBoZW4gRi4iLCJmYW1pbHkiOiJKYW5lIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IktldmluIEMuIiwiZmFtaWx5IjoiUm9zZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJZYXJvbiIsImZhbWlseSI6IkJl4oCZZXJpLVNobGV2aW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2FyYWggSC4iLCJmYW1pbHkiOiJCdXJuZXQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiRnJhbsOnb2lzIiwiZmFtaWx5IjoiQ2xheWVyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhlaWRydW4iLCJmYW1pbHkiOiJGZXVjaHRtYXlyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhhbnPigJBQZXRlciIsImZhbWlseSI6Ikdyb3NzYXJ0In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkRleHRlciBXLiIsImZhbWlseSI6Ikhvd2FyZCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJIZWF0aGVyIiwiZmFtaWx5IjoiTWFyaWFzaCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJKb3JkaSBEZWxnYWRvIiwiZmFtaWx5IjoiTWFydMOtbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJSZWJlY2NhIEwuIiwiZmFtaWx5IjoiTm9ydGgifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSXNhYmVsbGEgQS4iLCJmYW1pbHkiOiJPbGVrc3kifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUmFjaGVsIE0uIiwiZmFtaWx5IjoiUGlsbGEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW15IFAuIiwiZmFtaWx5IjoiU21hZ3VsYSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJSdWLDqW4iLCJmYW1pbHkiOiJTb21tYXJ1Z2EifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2FyYSBFLiIsImZhbWlseSI6IlN0ZWluZXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUGlldCIsImZhbWlseSI6IlZlcmJ1cmcifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiRGFuaWVsbGUiLCJmYW1pbHkiOiJXYWluIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikdlc2EgQS4iLCJmYW1pbHkiOiJXZXloZW5tZXllciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJDYXllbGFuIEMuIiwiZmFtaWx5IjoiQ2FyZXkifX1dLCJpZHMiOnsiaXNzbiI6WyIxMzU0LTEwMTMiLCIxMzY1LTI0ODYiXSwiaXNibiI6W10sImRvaSI6IjEwLjExMTEvZ2NiLjE3MDQ2IiwicG1pZCI6IjM4MjczNTM1Iiwib3BlbmFsZXgiOiJXNDM4OTM5MjUyOCIsImV4YSI6Imh0dHBzOi8vb25saW5lbGlicmFyeS53aWxleS5jb20vZG9pLzEwLjExMTEvZ2NiLjE3MDQ2In0sInRpdGxlIjoiQW5veGlhIGJlZ2V0cyBhbm94aWE6IEEgcG9zaXRpdmUgZmVlZGJhY2sgdG8gdGhlIGRlb3h5Z2VuYXRpb24gb2YgdGVtcGVyYXRlIGxha2VzIiwiYWJzdHJhY3QiOiJBYnN0cmFjdCBEZWNsaW5pbmcgb3h5Z2VuIGNvbmNlbnRyYXRpb25zIGluIHRoZSBkZWVwIHdhdGVycyBvZiBsYWtlcyB3b3JsZHdpZGUgcG9zZSBhIHByZXNzaW5nIGVudmlyb25tZW50YWwgYW5kIHNvY2lldGFsIGNoYWxsZW5nZS4gRXhpc3RpbmcgdGhlb3J5IHN1Z2dlc3RzIHRoYXQgbG93IGRlZXDigJB3YXRlciBkaXNzb2x2ZWQgb3h5Z2VuIChETykgY29uY2VudHJhdGlvbnMgY291bGQgdHJpZ2dlciBhIHBvc2l0aXZlIGZlZWRiYWNrIHRocm91Z2ggd2hpY2ggYW5veGlhIChpLmUuLCB2ZXJ5IGxvdyBETykgZHVyaW5nIGEgZ2l2ZW4gc3VtbWVyIGJlZ2V0cyBpbmNyZWFzaW5nbHkgc2V2ZXJlIG9jY3VycmVuY2VzIG9mIGFub3hpYSBpbiBmb2xsb3dpbmcgc3VtbWVycy4gU3BlY2lmaWNhbGx5LCBhbm94aWMgY29uZGl0aW9ucyBjYW4gcHJvbW90ZSBudXRyaWVudCByZWxlYXNlIGZyb20gc2VkaW1lbnRzLCB0aGVyZWJ5IHN0aW11bGF0aW5nIHBoeXRvcGxhbmt0b24gZ3Jvd3RoLCBhbmQgc3Vic2VxdWVudCBwaHl0b3BsYW5rdG9uIGRlY29tcG9zaXRpb24gY2FuIGZ1ZWwgaGV0ZXJvdHJvcGhpYyByZXNwaXJhdGlvbiwgcmVzdWx0aW5nIGluIGluY3JlYXNlZCBzcGF0aWFsIGV4dGVudCBhbmQgZHVyYXRpb24gb2YgYW5veGlhLiBIb3dldmVyLCB3aGlsZSB0aGUgaW5kaXZpZHVhbCByZWxhdGlvbnNoaXBzIGluIHRoaXMgZmVlZGJhY2sgYXJlIHdlbGwgZXN0YWJsaXNoZWQsIHRvIG91ciBrbm93bGVkZ2UsIHRoZXJlIGhhcyBub3QgYmVlbiBhIHN5c3RlbWF0aWMgYW5hbHlzaXMgd2l0aGluIG9yIGFjcm9zcyBsYWtlcyB0aGF0IHNpbXVsdGFuZW91c2x5IGRlbW9uc3RyYXRlcyBhbGwgb2YgdGhlIG1lY2hhbmlzbXMgbmVjZXNzYXJ5IHRvIHByb2R1Y2UgYSBwb3NpdGl2ZSBmZWVkYmFjayB0aGF0IHJlaW5mb3JjZXMgYW5veGlhLiBIZXJlLCB3ZSBjb21waWxlZCBkYXRhIGZyb20gNjU2IHdpZGVzcHJlYWQgdGVtcGVyYXRlIGxha2VzIGFuZCByZXNlcnZvaXJzIHRvIGFuYWx5emUgdGhlIHByb3Bvc2VkIGFub3hpYSBiZWdldHMgYW5veGlhIGZlZWRiYWNrLiBMYWtlcyBpbiB0aGUgZGF0YXNldCBzcGFuIGEgYnJvYWQgcmFuZ2Ugb2Ygc3VyZmFjZSBhcmVhICgx4oCTMTI2LDkwOSBoYSksIG1heGltdW0gZGVwdGggKDbigJMzNzAgbSksIGFuZCBtb3JwaG9tZXRyeSwgd2l0aCBhIG1lZGlhbiB0aW1l4oCQc2VyaWVzIGR1cmF0aW9uIG9mIDMwIHllYXJzIGF0IGVhY2ggbGFrZS4gVXNpbmcgbGluZWFyIG1peGVkIG1vZGVscywgd2UgZm91bmQgc3VwcG9ydCBmb3IgZWFjaCBvZiB0aGUgcG9zaXRpdmUgZmVlZGJhY2sgcmVsYXRpb25zaGlwcyBiZXR3ZWVuIGFub3hpYSwgcGhvc3Bob3J1cyBjb25jZW50cmF0aW9ucywgY2hsb3JvcGh5bGwgYSBjb25jZW50cmF0aW9ucywgYW5kIG94eWdlbiBkZW1hbmQgYWNyb3NzIHRoZSA2NTbigJBsYWtlIGRhdGFzZXQuIExpa2V3aXNlLCB3ZSBmb3VuZCBmdXJ0aGVyIHN1cHBvcnQgZm9yIHRoZXNlIHJlbGF0aW9uc2hpcHMgYnkgYW5hbHl6aW5nIHRpbWXigJBzZXJpZXMgZGF0YSBmcm9tIGluZGl2aWR1YWwgbGFrZXMuIE91ciByZXN1bHRzIGluZGljYXRlIHRoYXQgdGhlIHN0cmVuZ3RoIG9mIHRoZXNlIGZlZWRiYWNrIHJlbGF0aW9uc2hpcHMgbWF5IHZhcnkgd2l0aCBsYWtl4oCQc3BlY2lmaWMgY2hhcmFjdGVyaXN0aWNzOiBGb3IgZXhhbXBsZSwgd2UgZm91bmQgdGhhdCBzdXJmYWNlIHBob3NwaG9ydXMgY29uY2VudHJhdGlvbnMgd2VyZSBtb3JlIHBvc2l0aXZlbHkgYXNzb2NpYXRlZCB3aXRoIGNobG9yb3BoeWxsIGEgaW4gaGlnaOKAkHBob3NwaG9ydXMgbGFrZXMsIGFuZCBveHlnZW4gZGVtYW5kIGhhZCBhIHN0cm9uZ2VyIGluZmx1ZW5jZSBvbiB0aGUgZXh0ZW50IG9mIGFub3hpYSBpbiBkZWVwIGxha2VzLiBUYWtlbiB0b2dldGhlciwgdGhlc2UgcmVzdWx0cyBzdXBwb3J0IHRoZSBleGlzdGVuY2Ugb2YgYSBwb3NpdGl2ZSBmZWVkYmFjayB0aGF0IGNvdWxkIG1hZ25pZnkgdGhlIGVmZmVjdHMgb2YgY2xpbWF0ZSBjaGFuZ2UgYW5kIG90aGVyIGFudGhyb3BvZ2VuaWMgcHJlc3N1cmVzIGRyaXZpbmcgdGhlIGRldmVsb3BtZW50IG9mIGFub3hpYSBpbiBsYWtlcyBhcm91bmQgdGhlIHdvcmxkLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMTExL2djYi4xNzA0NiIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9vbmxpbmVsaWJyYXJ5LndpbGV5LmNvbS9kb2kvcGRmZGlyZWN0LzEwLjExMTEvZ2NiLjE3MDQ2IiwiaXNzdWVkRGF0ZSI6IjIwMjMtMTItMDYiLCJhY2Nlc3NlZERhdGUiOiIyMDI2LTAyIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJHbG9iYWwgQ2hhbmdlIEJpb2xvZ3kiLCJpc3N1ZSI6MSwidm9sdW1lIjozMCwiaW1wYWN0RmFjdG9yIjoiNy40NSJ9LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJXaWxleSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MTE3LCJjaXRlZEJ5Q291bnQiOjI5fX0sInF1b3RlIjoiR2xvYmFsIENoYW5nZSBCaW9sb2d5IFxcfCBFbnZpcm9ubWVudGFsIENoYW5nZSBKb3VybmFsIFxcfCBXaWxleSBPbmxpbmUgTGlicmFyeSAuLi4gQ2xvc2UgQ29va2llIFByZWZlcmVuY2VzXG5cbiBHbG9iYWwgQ2hhbmdlIEJpb2xvZ3kgLi4uICMgQW5veGlhIGJlZ2V0cyBhbm94aWE6IEEgcG9zaXRpdmUgZmVlZGJhY2sgdG8gdGhlIGRlb3h5Z2VuYXRpb24gb2YgdGVtcGVyYXRlIGxha2VzXG5cbiBBYmlnYWlsIFMuIEwuIExld2lzICwgLi4uIGh0dHBzOi8vZG9pLm9yZy8xMC4xMTExL2djYi4xNzA0NiBcblxuIyMgQWJzdHJhY3QgRGVjbGluaW5nIG94eWdlbiBjb25jZW50cmF0aW9ucyBpbiB0aGUgZGVlcCB3YXRlcnMgb2YgbGFrZXMgd29ybGR3aWRlIHBvc2UgYSBwcmVzc2luZyBlbnZpcm9ubWVudGFsIGFuZCBzb2NpZXRhbCBjaGFsbGVuZ2UuIEV4aXN0aW5nIHRoZW9yeSBzdWdnZXN0cyB0aGF0IGxvdyBkZWVwLXdhdGVyIGRpc3NvbHZlZCBveHlnZW4gKERPKSBjb25jZW50cmF0aW9ucyBjb3VsZCB0cmlnZ2VyIGEgcG9zaXRpdmUgZmVlZGJhY2sgdGhyb3VnaCB3aGljaCBhbm94aWEgKGkuZS4sIHZlcnkgbG93IERPKSBkdXJpbmcgYSBnaXZlbiBzdW1tZXIgYmVnZXRzIGluY3JlYXNpbmdseSBzZXZlcmUgb2NjdXJyZW5jZXMgb2YgYW5veGlhIGluIGZvbGxvd2luZyBzdW1tZXJzLiBTcGVjaWZpY2FsbHksIGFub3hpYyBjb25kaXRpb25zIGNhbiBwcm9tb3RlIG51dHJpZW50IHJlbGVhc2UgZnJvbSBzZWRpbWVudHMsIHRoZXJlYnkgLi4uIGZyZXNod2F0ZXIgZWNvc3lzdGVtcywgYXMgbG93IERPIGNvbmRpdGlvbnMgY2FuIGRlY3JlYXNlIGhhYml0YXQgc3VpdGFiaWxpdHkgZm9yIG51bWVyb3VzIGFxdWF0aWMgb3JnYW5pc21zIChlLmcuLCAuLi4gZXTCoGFsLizCoCAyMDIxICkuIEFzIGZyZXNod2F0ZXJzIGFyZSBjcml0aWNhbCBlY29zeXN0ZW1zIGZvciBkcmlua2luZyB3YXRlciwgcmVjcmVhdGlvbiwgaXJyaWdhdGlvbiwgYW5kIC4uLiBSZXluYXVkICYiLCJleHRlcm5hbENhY2hlSWQiOiI0OTA3ZTUzYi1mNDA2LTMzNWItYmJkZi05NGVjYjZkMWIwNzMifV0%3D. The persistence of these anaerobic conditions is further exacerbated by thermal stratification, which creates a density barrier that inhibits the vertical transport of oxygen from surface layers to bottom waters, thereby accelerating hypoxic processes and intensifying methane production (Ji et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkRhb2JpbiIsImZhbWlseSI6IkppIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllhbnhpbmciLCJmYW1pbHkiOiJIYW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTGlhbmdob25nIiwiZmFtaWx5IjoiTG9uZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJYaWFva2FuZyIsImZhbWlseSI6IlhpbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJIdWl5YW4iLCJmYW1pbHkiOiJYdSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTaXFpYW4iLCJmYW1pbHkiOiJRaXUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSmlhbmdodWFpIiwiZmFtaWx5IjoiTWVuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJYaW5neGluZyIsImZhbWlseSI6IlpoYW8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiWWFuYW4iLCJmYW1pbHkiOiJIdWFuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJEZWZ1IiwiZmFtaWx5IjoiTGl1In19XSwiaWRzIjp7Imlzc24iOlsiMjI5Ni03MDFYIl0sImlzYm4iOltdLCJkb2kiOiIxMC4zMzg5L2Zldm8uMjAyMy4xMjk3MDQ3Iiwib3BlbmFsZXgiOiJXNDM5MDYxMDY2OCIsIm9wZW5BY2Nlc3NJZCI6IjEwMjg2MjU3NDMyMjQ0MDQ2OTgxIn0sInRpdGxlIjoiSHlwb3hpYSBhbmQgaXRzIGZlZWRiYWNrIHJlc3BvbnNlIHRvIGFsZ2FsIGJsb29tcyBhbmQgQ0g0IGVtaXNzaW9ucyBpbiBzdWJ0cm9waWNhbCByZXNlcnZvaXJzIiwiYWJzdHJhY3QiOiJBbnRocm9wb2dlbmljIG51dHJpZW50IGlucHV0IHN0aW11bGF0ZXMgc3VyZmFjZSBldXRyb3BoaWNhdGlvbiBhbmQgYWxnYWwgYmxvb21zIGFuZCB0aHVzIGV4YWNlcmJhdGVzIG94eWdlbiBkZXBsZXRpb24gYW5kIGh5cG94aWEgaW4gYXF1YXRpYyBlY29zeXN0ZW1zLiBIb3dldmVyLCBleGlzdGluZyBzdHVkaWVzIGhhdmUgcGFpZCBsaXR0bGUgYXR0ZW50aW9uIG9uIHRoZSB1bmRlcnN0YW5kaW5nIG9mIHRoZSBmZWVkYmFjayByZWxhdGlvbnNoaXBzIGFtb25nIG94eWdlbiBjb25zdW1wdGlvbiwgYWxnYWwgc3VjY2Vzc2lvbiBhbmQgZ3JlZW5ob3VzZSBnYXMgZWZmZWN0IGFmdGVyIHJpdmVyIGRhbW1pbmcsIHBhcnRpY3VsYXJseSBpbiBUaHJlZSBHb3JnZXMgUmVzZXJ2b2lyLiBUaGlzIGZpZWxkIHdvcmsgaW52ZXN0aWdhdGVkIHRoZSB0aGVybWFsIHN0cmF0aWZpY2F0aW9uLCB3YXRlciBxdWFsaXR5IGFuZCBwaHl0b3BsYW5rdG9uIGluIGZvdXIgdHJpYnV0YXJpZXMgKFBlbmd4aSBCYXkgKFBYQiksIERhbmluZyBCYXkgKEROQiksIFNoZW5ub25nIEJheSAoU05CKSwgYW5kIFhpYW5neGkgQmF5IChYWEIpKSBvZiB0aGUgVGhyZWUgR29yZ2VzIFJlc2Vydm9pciAoVEdSKSBpbiBzcHJpbmcgKEFwcmlsIDI1LTI4LCAyMDIyKSwgYW5kIGV4cGxvcmVkIHRoZSBkcml2ZXJzIG9mIGh5cG94aWEgYW5kIGl0cyBmZWVkYmFjayByZXNwb25zZSB0byBhbGdhbCBibG9vbXMgYW5kIG1ldGhhbmUgKENIIDQgKSBlbWlzc2lvbnMuIFRoZSByZXN1bHRzIHNob3dlZCB0aGF0IGR1cmluZyB0aGUgb2JzZXJ2YXRpb24gcGVyaW9kLCBETywgdGhlcm1hbCBzdHJhdGlmaWNhdGlvbiwgYW5kIGFsZ2FsIGJsb29tcyB3ZXJlIG1vcmUgc2V2ZXJlIGluIFBYQi4gVGhlIGhpZ2ggQ2hlbWljYWwgT3h5Z2VuIERlbWFuZCAoQ09EIE1uICkgKDExLjI3IG1n4oCiTCAtMSApIGFuZCBEaXNzb2x2ZWQgT3JnYW5pYyBDYXJib24gKERPQykgY29udGVudCAoNy41NiBtZ%2BKAokwgLTEgKSBpbiBQWEIgd2VyZSB0aGUgbWFpbiBjYXVzZXMgb2YgdGhlIHByb2dyZXNzaXZlIGh5cG94aWEuIEZ1cnRoZXJtb3JlLCBzdHJvbmdlciBkZW5zaXR5IHN0cmF0aWZpY2F0aW9uIGluIFBYQiBpbmhpYml0ZWQgdGhlIHZlcnRpY2FsIHN1cHBseSBvZiBETyBhbmQgYWNjZWxlcmF0ZWQgYm90dG9tIGh5cG94aWEuIEJ1dCB0aGUgZG93bnNsb3BlIGRlbnNpdHkgY3VycmVudCByZXBsZW5pc2hlZCBib3R0b20gRE8gaW4gdGhlIHVwcGVyIHJlYWNoZXMgb2YgdGhlIEROQiwgU05CLCBhbmQgWFhCLiBUaGUgQ0g0IGNvbmNlbnRyYXRpb24gd2FzIGdlbmVyYWxseSBoaWdoZXIgaW4gaHlwb3hpYSB6b25lLCBlc3BlY2lhbGx5IGluIHdoZXJlIGN5YW5vYmFjdGVyaWEgYXJlIHRoZSBkb21pbmFudCBzcGVjaWVzIChlZy4sIFBYMDMgYW5kIFBYMDQpLiBEYW0gY29uc3RydWN0aW9uIGxlZCB0byB0aGUgc3VjY2Vzc2lvbiBvZiBhbGdhbC1kb21pbmFudCBzcGVjaWVzLCBleGVydGluZyBhIHNpZ25pZmljYW50IGluZmx1ZW5jZSBvbiB0aGUgcml2ZXIgY2FyYm9uIGN5Y2xlLiBUaGUgZXhwYW5zaW9uIG9mIGN5YW5vYmFjdGVyaWFsIGJsb29tcyBhbmQgdGhlIGNvbnNlcXVlbmNlIG9mIG1ldGFib2xpYyBzdWJzdHJhdGVzIGluY3JlYXNlIGFyZSBncmFkdWFsbHkgaW5jcmVhc2luZyBveHlnZW4gZGVwcml2YXRpb24sIGV2ZW4gaHlwb3hpYSwgYW5kIGVuaGFuY2luZyBDSCA0IHByb2R1Y3Rpb24gaW4gcml2ZXJzLiBUaGVzZSBmaW5kaW5ncyBzdWdnZXN0IHRoYXQgdGhlIHN1Y2Nlc3Npb24gb2YgYWxnYWwtZG9taW5hbnQgc3BlY2llcyBjYXVzZWQgYnkgcml2ZXIgZGFtbWluZyBoYXMgYSBwcm9ub3VuY2VkIHBvc2l0aXZlIGZlZWRiYWNrIGVmZmVjdCBvbiByZXNlcnZvaXIgaHlwb3hpYSBwcm9jZXNzZXMgYW5kIGdyZWVuaG91c2UgZWZmZWN0LiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzg5L2Zldm8uMjAyMy4xMjk3MDQ3IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5mcm9udGllcnNpbi5vcmcvYXJ0aWNsZXMvMTAuMzM4OS9mZXZvLjIwMjMuMTI5NzA0Ny9wZGY%2FaXNQdWJsaXNoZWRWMj1GYWxzZSIsImlzc3VlZERhdGUiOiIyMDI0LTAxLTA1IiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiRnJvbnRpZXJzIGluIEVjb2xvZ3kgYW5kIEV2b2x1dGlvbiIsInZvbHVtZSI6MTEsImltcGFjdEZhY3RvciI6IjQuNzgifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRnJvbnRpZXJzIE1lZGlhIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo0MCwiY2l0ZWRCeUNvdW50Ijo1fX0sInF1b3RlIjoiVGhlcmVmb3JlLCB1bmRlciB0aGUgYmFja2dyb3VuZCBvZiBjb250aW51b3VzIGN5YW5vYmFjdGVyaWFsIGJsb29tcywgaHlwb3hpYyBhbmQgYW5hZXJvYmljIHBoZW5vbWVuYSBpbiB3YXRlcnMgaGF2ZSBiZWNvbWUgYW5vdGhlciBlbWVyZ2luZyB3YXRlciBlY29sb2dpY2FsIGVudmlyb25tZW50YWwgcHJvYmxlbSBpbiB0aGUgdHJpYnV0YXJ5IGJheXMgb2YgdGhlIFRHUi4gSW4gdGhpcyBzdHVkeSwgdGhlIGNsb3NlciBhIHRyaWJ1dGFyeSBiYXkgd2FzIHRvIHRoZSB1cHBlciByZWFjaGVzLCB0aGUgbW9yZSBwcm9ub3VuY2VkIHRoZSBkZWdyZWUgb2YgaHlwb3hpYSwgYWxnYWwgYmxvb21zIGFuZCB0aGUgaGlnaGVyIHRoZSBjb25jZW50cmF0aW9ucyBvZiBDSDQgYW5kIENPMiBpbiB0aGUgd2F0ZXJzLiBQbGFua3RvbmljIGFsZ2FlIGFyZSBhbiBpbXBvcnRhbnQgZmFjdG9yIGFmZmVjdGluZyB0aGUgY2FyYm9uIGN5Y2xlIG9mIGEgcmVzZXJ2b2lyIGVjb3N5c3RlbS4gSW4gdGhpcyB0eXBlIG9mIGVjb3N5c3RlbS4gV2hlbiB0aGUgYWxnYWUgZGVjbGluZSBhbmQgc2luaywgdGhleSByZWxlYXNlIGZyZXNoIGFsZ2FsIG9yZ2FuaWMgbWF0dGVyIChBT00sIEZpZ3VyZSAxMCwg4pGgKSBpbnRvIHRoZSB3YXRlcnMsIGFuZCB0aGUgb3h5Z2VuLWNvbnN1bWluZyBkZWNvbXBvc2l0aW9uIG9mIG9yZ2FuaWMgbWF0dGVyIGFjY2VsZXJhdGVzIHRoZSBjb25zdW1wdGlvbiBvZiBETyBpbiB0aGUgc3Vic3VyZmFjZSBhbmQgYm90dG9tIHdhdGVycyAoRmlndXJlIDEwLCDikaEpLiBBIHByZXZpb3VzIHN0dWR5IChaaGFuZywgMjAxNSkgc2hvd2VkIHRoYXQgdGhlIGRlY2xpbmUgaW4gYWxnYWwgYmxvb21zIG5vdCBvbmx5IHByb21vdGVkIHRoZSBmb3JtYXRpb24gb2YgYW4gYW5hZXJvYmljIGVudmlyb25tZW50IGJ1dCBhbHNvIHByb3ZpZGVkIGFidW5kYW50IHJlYWN0aW9uIHN1YnN0cmF0ZXMgZm9yIG1ldGhhbm9nZW5pYyBtaWNyb29yZ2FuaXNtcy4gQWRkaXRpb25hbGx5LCB0aGUgZGVuc2l0eSBkaWZmZXJlbmNlIGNhdXNlZCBieSB3YXRlciB0ZW1wZXJhdHVyZSBzdHJhdGlmaWNhdGlvbiBoaW5kZXJzIHRoZSB2ZXJ0aWNhbCB0cmFuc3BvcnQgYW5kIHN1cHBseSBvZiBETywgYWNjZWxlcmF0aW5nIHRoZSBoeXBveGljIGFuZCBhbmFlcm9iaWMgcHJvY2Vzc2VzIGluIHRoZSBib3R0b20gd2F0ZXJzLCBUaGlzIGludGVuc2lmaWVzIHRoZSBwcm9kdWN0aW9uIG9mIENINCB1bmRlciBoeXBveGljIGFuZCBhbmFlcm9iaWMgY29uZGl0aW9ucyAoRmlndXJlIDEwLCDikaIpLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MTEsImJib3giOnsidG9wIjoxOS4xNDc0NTkwMzAxNTEzNjcsImxlZnQiOjUxLjIwOTM0Mjk1NjU0Mjk3LCJ3aWR0aCI6NDAuNDkwMDYyNzEzNjIzMDUsImhlaWdodCI6MS4zNDc1NDk2NzY4OTUxNDE2fX0seyJwYWdlTnVtYmVyIjoxMSwiYmJveCI6eyJ0b3AiOjIwLjgwMzA3MTk3NTcwODAwOCwibGVmdCI6NTEuMjA5MjI4NTE1NjI1LCJ3aWR0aCI6NDAuNDgzNTA5MDYzNzIwNywiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjExLCJiYm94Ijp7InRvcCI6MjIuMjg0OTI3MzY4MTY0MDYyLCJsZWZ0Ijo1MS4yMDkyMjg1MTU2MjUsIndpZHRoIjo0MC40ODA3OTY4MTM5NjQ4NDQsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMSwiYmJveCI6eyJ0b3AiOjIzLjc3MzUxMTg4NjU5NjY4LCJsZWZ0Ijo1MS4yMDkyMjg1MTU2MjUsIndpZHRoIjo0MC40ODMxOTYyNTg1NDQ5MiwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjExLCJiYm94Ijp7InRvcCI6MjUuMjU1MzY3Mjc5MDUyNzM0LCJsZWZ0Ijo1MS4yMDkyMjg1MTU2MjUsIndpZHRoIjo1LjU5ODQyMzQ4MDk4NzU0OSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjExLCJiYm94Ijp7InRvcCI6MjYuNzQzOTUzNzA0ODMzOTg0LCJsZWZ0Ijo1My41NTE2MzE5Mjc0OTAyMzQsIndpZHRoIjozOC4xMzUxOTI4NzEwOTM3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjExLCJiYm94Ijp7InRvcCI6MjguMjI1ODA5MDk3MjkwMDQsImxlZnQiOjUxLjIwOTIyODUxNTYyNSwid2lkdGgiOjQwLjQ5MDAwOTMwNzg2MTMzLCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTEsImJib3giOnsidG9wIjoyOS43MTQzOTM2MTU3MjI2NTYsImxlZnQiOjUxLjIwOTIyODUxNTYyNSwid2lkdGgiOjQwLjQ4MDk0OTQwMTg1NTQ3LCJoZWlnaHQiOjEuMzU1ODYwMjMzMzA2ODg0OH19LHsicGFnZU51bWJlciI6MTEsImJib3giOnsidG9wIjozMS4xOTY0NjA3MjM4NzY5NTMsImxlZnQiOjUxLjIwOTIyODUxNTYyNSwid2lkdGgiOjQwLjQ4MTM4MDQ2MjY0NjQ4NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjExLCJiYm94Ijp7InRvcCI6MzIuNjg1MDUwOTY0MzU1NDcsImxlZnQiOjUxLjIwOTIyODUxNTYyNSwid2lkdGgiOjQwLjQ3OTU3MjI5NjE0MjU4LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTEsImJib3giOnsidG9wIjozNC4xNjY5MDQ0NDk0NjI4OSwibGVmdCI6NTEuMjA5MjI4NTE1NjI1LCJ3aWR0aCI6NDAuNDg2NTE1MDQ1MTY2MDE2LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTEsImJib3giOnsidG9wIjozNS42NTU0OTA4NzUyNDQxNCwibGVmdCI6NTEuMjA5MjI4NTE1NjI1LCJ3aWR0aCI6NDAuNDkzMDM0MzYyNzkyOTcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMSwiYmJveCI6eyJ0b3AiOjM3LjEzNzM0ODE3NTA0ODgzLCJsZWZ0Ijo1MS4yMDkyMjg1MTU2MjUsIndpZHRoIjo0MC40ODMzNjAyOTA1MjczNDQsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMSwiYmJveCI6eyJ0b3AiOjM4LjYyNTkzNDYwMDgzMDA4LCJsZWZ0Ijo1MS4yMDkyMjg1MTU2MjUsIndpZHRoIjo0MC40Nzg1Mzg1MTMxODM1OTQsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMSwiYmJveCI6eyJ0b3AiOjQwLjEwNzc4ODA4NTkzNzUsImxlZnQiOjUxLjIwOTA3NTkyNzczNDM3NSwid2lkdGgiOjQwLjQ4OTA5NzU5NTIxNDg0NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjExLCJiYm94Ijp7InRvcCI6NDEuNTk2Mzc4MzI2NDE2MDE2LCJsZWZ0Ijo1MS4yMDkwNzU5Mjc3MzQzNzUsIndpZHRoIjo0MC40ODkwOTc1OTUyMTQ4NDQsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMSwiYmJveCI6eyJ0b3AiOjQzLjA3ODIyNzk5NjgyNjE3LCJsZWZ0Ijo1MS4yMDkwNzU5Mjc3MzQzNzUsIndpZHRoIjo0MC40ODA3ODUzNjk4NzMwNSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjExLCJiYm94Ijp7InRvcCI6NDQuNTY2ODE0NDIyNjA3NDIsImxlZnQiOjUxLjIwOTA3NTkyNzczNDM3NSwid2lkdGgiOjQwLjQ3ODM2MzAzNzEwOTM3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjExLCJiYm94Ijp7InRvcCI6NDYuMDQ4NjY3OTA3NzE0ODQ0LCJsZWZ0Ijo1MS4yMDkwNzU5Mjc3MzQzNzUsIndpZHRoIjo0MC40ODU2MDMzMzI1MTk1MywiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjExLCJiYm94Ijp7InRvcCI6NDcuNTM3MjU4MTQ4MTkzMzYsImxlZnQiOjUxLjIwOTA3NTkyNzczNDM3NSwid2lkdGgiOjQwLjQ4ODY1ODkwNTAyOTMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMSwiYmJveCI6eyJ0b3AiOjQ5LjAxOTExMTYzMzMwMDc4LCJsZWZ0Ijo1MS4yMDkwNzU5Mjc3MzQzNzUsIndpZHRoIjo0MC40ODU4ODk0MzQ4MTQ0NSwiaGVpZ2h0IjoxLjM2Mjc0MjA2NjM4MzM2MTh9fSx7InBhZ2VOdW1iZXIiOjExLCJiYm94Ijp7InRvcCI6NTAuNTA4MDQxMzgxODM1OTQsImxlZnQiOjUxLjIwOTMyMzg4MzA1NjY0LCJ3aWR0aCI6NDAuNDg2MjUxODMxMDU0NjksImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX1dfSwiY2h1bmtJZCI6IjI2NGMxNmIxLTcyNWEtNDA4OC04ZTUyLTM0ZWYxZDgwYTdmZiIsImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEyIn19XQ%3D%3D.

Table 3: Ecological Consequences of Harmful Algal Blooms

| Consequence | Description |
| --- | --- |
| Oxygen Depletion | Decomposition of algal biomass leading to hypoxia and anoxia |
| Biodiversity Loss | Decimation of fish stocks, zooplankton, and benthic organisms |
| Toxin Production | Release of cyanotoxins endangering human and animal health |
| Habitat Degradation | Smothering of benthic habitats and loss of macrophyte cover |

8. 8. Public Health Risks: The Threat of Cyanotoxins

Cyanobacterial harmful algal blooms in Indian lakes pose profound public health threats through the production of potent cyanotoxins, including hepatotoxins like microcystins and neurotoxins such as anatoxins and saxitoxins. These toxins are secondary metabolites synthesized by dominant bloom-forming genera like Microcystis aeruginosa and Dolichospermum spp., which thrive under eutrophic conditions exacerbated by nutrient pollution and warming climates (Meerhoff et al., 2022; Paerl et al., 2019)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik0uIiwiZmFtaWx5IjoiTWVlcmhvZmYifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSi4iLCJmYW1pbHkiOiJBdWRldCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJULkEuIiwiZmFtaWx5IjoiRGF2aWRzb24ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTC4gRGUiLCJmYW1pbHkiOiJNZWVzdGVyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlMuIiwiZmFtaWx5IjoiSGlsdCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTLiIsImZhbWlseSI6Iktvc3RlbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJaYWlodWEiLCJmYW1pbHkiOiJMaXUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTi4iLCJmYW1pbHkiOiJNYXp6ZW8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSGFucyBXLiIsImZhbWlseSI6IlBhZXJsIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik0uIiwiZmFtaWx5IjoiU2NoZWZmZXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiRS4iLCJmYW1pbHkiOiJKZXBwZXNlbiJ9fV0sImlkcyI6eyJpc3NuIjpbXSwiaXNibiI6W10sImRvaSI6IjEwLjE3NjE1Lzd2ZTUtdzAxNiIsIm9wZW5hbGV4IjoiVzQzMTg0MTU1NzcifSwidGl0bGUiOiJGZWVkYmFjayBiZXR3ZWVuIGNsaW1hdGUgY2hhbmdlIGFuZCBldXRyb3BoaWNhdGlvbjogcmV2aXNpdGluZyB0aGUgYWxsaWVkIGF0dGFjayBjb25jZXB0IGFuZCBob3cgdG8gc3RyaWtlIGJhY2siLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTc2MTUvN3ZlNS13MDE2IiwicmVzb3VyY2VVcmwiOiJodHRwczovL2Nkci5saWIudW5jLmVkdS9kb3dubG9hZHMvaDk4OXJjOTM4IiwiaXNzdWVkRGF0ZSI6IjIwMjItMDEtMDEiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTEwIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJDYXJvbGluYSBEaWdpdGFsIFJlcG9zaXRvcnkgKFVuaXZlcnNpdHkgb2YgTm9ydGggQ2Fyb2xpbmEgYXQgQ2hhcGVsIEhpbGwpIiwiaW1wYWN0RmFjdG9yIjoiMC4xMCJ9LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJVbml2ZXJzaXR5IG9mIE5vcnRoIENhcm9saW5hIGF0IENoYXBlbCBIaWxsIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjowLCJjaXRlZEJ5Q291bnQiOjN9fSwicXVvdGUiOiJXaXRoIGNsaW1hdGUgd2FybWluZywgdGhlIGRpcmVjdCBhbmQgaW5kaS1yZWN0IGVmZmVjdHMgb2YgaW5jcmVhc2VkIGxha2UgdGVtcGVyYXR1cmVzIHdpbGwgbGlrZWx5IGRyaXZlIGNoYW5nZXMgaW4gY3lhbm9iYWN0ZXJpYWwgdGF4YSBhbmQgdGhlaXIgcHJvZHVjLXRpb24gb2YgY3lhbm9iYWN0ZXJpYWwgdG94aW5zLCBsaWtlbHkgaW5jbHVkaW5nIHByb21vdC1pbmcgdGhlIHNlbGVjdGlvbiBvZiBhIGZldyBNaWNyb2N5c3RpcyBzcGVjaWVzIG9yIHN0cmFpbnMgcHJvZHVjaW5nIGhpZ2hseSB0b3hpYyBtaWNyb2N5c3Rpbi1MUiAoTWFudHpvdWtpIGV0IGFsLiAyMDE4KS4gRGVzcGl0ZSB2YXJpYXRpb25zIGFtb25nIGxha2VzIGluIGEgd2F0ZXItc2NhcGUsIGEgcmVnaW9uYWwgaW5jcmVhc2UgaW4gbWljcm9jeXN0aW4gbWF4aW1hIGFuZCBkdXJhdGlvbiBvZiBibG9vbXMgaGFzIGFsc28gYmVlbiBjbGVhcmx5IGV4cGxhaW5lZCBieSBpbmNyZWFzaW5nIHRlbXBlcmF0dXJlIGluIGNlbnRyYWwgTm9ydGggQW1lcmljYSAoSGF5ZXMgZXQgYWwuIDIwMjApLCB3aXRoIGluY3JlYXNpbmcgcmlza3MgdG8gaHVtYW4gaGVhbHRoLiBHcm93aW5nIGV2aWRlbmNlIGluZGljYXRlcyB0aGF0IGN5YW5vYmFjdGVyaWFsIGRvbWluYW5jZSB3aWxsIGV4ZXJ0IGEgcG9zaXRpdmUgZmVlZGJhY2sgd2l0aCBjbGltYXRlIHdhcm1pbmcgKEhhdmVucyBldCBhbC4gMjAxNiwgUGFlcmwgZXQgYWwuIDIwMjApLiBNYW55IGN5YW5vYmFjdGVyaWFsIHNwZWNpZXMgcHJvbGlmZXJhdGUgb24gdGhlIGxha2Ugc3VyZmFjZSwgYWJzb3JiaW5nIGxpZ2h0LCBlbmhhbmNpbmcgbG9jYWxpemVkIHRlbXBlcmF0dXJlIGluY3JlYXNlcywgYW5kIGluY3JlYXNpbmcgdGhlcm1hbCBzdHJhdGlmaWNhdGlvbiBwb3RlbnRpYWwgKEthaHJ1IGV0IGFsLiAxOTkzKS4gQ3lhbm9iYWN0ZXJpYSBwcm9tb3RlIHRoZSByZWxlYXNlIG9mIEdIR3MsIHBhcnRpY3VsYXJseSBDSDQgKFlhbiBldCBhbC4gMjAxNyksIGR1cmluZyB0aGUgZGVjb21wb3NpdGlvbiBmb2xsb3dpbmcgdGhlIGNvbGxhcHNlIG9mIGEgYmxvb20uIEN5YW5vYmFjdGVyaWEgYWxzbyBwcm9tb3RlIHJlbWluZXJhbGl6YXRpb24gb2YgbnV0cmktZW50cyBkdXJpbmcgYmxvb21zLCBsZWFkaW5nIHRvIG11bHRpcGxlIGJsb29tcyBpbiBhIHNpbmdsZSBzZWFzb24uIFVuZGVyIGV1dHJvcGhpYyBhbmQgd2FybWluZyBjb25kaS10aW9ucywgY3lhbm9iYWN0ZXJpYSBkb21pbmFuY2UgbWF5IHRodXMgYmUgYSBzZWxmLXBlcnBldHVhdGluZyBwcm9waGVjeSAoRmlnLiAyIGFuZCAzKS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo2Ny4wNjM0MDAyNjg1NTQ2OSwibGVmdCI6NTEuNDc4ODAxNzI3Mjk0OTIsIndpZHRoIjozOC42NzM0MjM3NjcwODk4NDQsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6NjguNjk4NTQ3MzYzMjgxMjUsImxlZnQiOjUxLjQ3ODgwMTcyNzI5NDkyLCJ3aWR0aCI6MzguNjc5NDk2NzY1MTM2NzIsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6NzAuMzMzNjg2ODI4NjEzMjgsImxlZnQiOjUxLjQ3ODgwMTcyNzI5NDkyLCJ3aWR0aCI6MzguNjc3NTgxNzg3MTA5Mzc1LCJoZWlnaHQiOjEuNDcxMTAxNTIyNDQ1Njc4N319LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjcxLjk2ODgyNjI5Mzk0NTMxLCJsZWZ0Ijo1MS40Nzg4MDE3MjcyOTQ5Miwid2lkdGgiOjM4LjY3OTEwMzg1MTMxODM2LCJoZWlnaHQiOjEuNDcxMTAxNTIyNDQ1Njc4N319LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjczLjYxMDU3MjgxNDk0MTQsImxlZnQiOjUxLjQ3ODgwMTcyNzI5NDkyLCJ3aWR0aCI6MzguNjc0NDI3MDMyNDcwNywiaGVpZ2h0IjoxLjQ3Mzc0Njg5NTc5MDF9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo3NS4yNDcwMzk3OTQ5MjE4OCwibGVmdCI6NTEuNDc4ODA5MzU2Njg5NDUsIndpZHRoIjozOC42NzQyNjMwMDA0ODgyOCwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo3Ni44ODIxODY4ODk2NDg0NCwibGVmdCI6NTEuNDc4ODA5MzU2Njg5NDUsIndpZHRoIjozOC42ODExNzE0MTcyMzYzMywiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo3OC41MjUyNTMyOTU4OTg0NCwibGVmdCI6NTEuNDc4ODA5MzU2Njg5NDUsIndpZHRoIjozOC42NzU2NjI5OTQzODQ3NjYsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODAuMTYwNDAwMzkwNjI1LCJsZWZ0Ijo1MS40Nzg4MDkzNTY2ODk0NSwid2lkdGgiOjM4LjY4MjAyOTcyNDEyMTA5NCwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo4MS43OTU1Mzk4NTU5NTcwMywibGVmdCI6NTEuNDc4ODA5MzU2Njg5NDUsIndpZHRoIjozOC42NzQ5ODM5NzgyNzE0ODQsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODMuNDM4NjEzODkxNjAxNTYsImxlZnQiOjUxLjQ3ODgwOTM1NjY4OTQ1LCJ3aWR0aCI6MzguNjc2MTg1NjA3OTEwMTU2LCJoZWlnaHQiOjEuNDg2OTczNTI0MDkzNjI4fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODUuMDg5NjMwMTI2OTUzMTIsImxlZnQiOjUxLjQ3ODgwOTM1NjY4OTQ1LCJ3aWR0aCI6NC42ODI4MDUwNjEzNDAzMzIsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODYuNzI0Nzc3MjIxNjc5NjksImxlZnQiOjUzLjQ0MTE3NzM2ODE2NDA2LCJ3aWR0aCI6Ni4xNDAzNTU1ODcwMDU2MTUsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODYuNzI0ODkxNjYyNTk3NjYsImxlZnQiOjYwLjk5NDg3Njg2MTU3MjI2Niwid2lkdGgiOjYuMDM2OTkyNTQ5ODk2MjQsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODYuNzI0ODkxNjYyNTk3NjYsImxlZnQiOjY4LjQ0NjUyNTU3MzczMDQ3LCJ3aWR0aCI6Ni4xNTA3MDU4MTQzNjE1NzIsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODYuNzI0ODkxNjYyNTk3NjYsImxlZnQiOjc2LjAwOTgyNjY2MDE1NjI1LCJ3aWR0aCI6Mi43MjczMzMzMDcyNjYyMzU0LCJoZWlnaHQiOjEuNDcxMTAxNTIyNDQ1Njc4N319LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjg2LjcyNDg5MTY2MjU5NzY2LCJsZWZ0Ijo4MC4xNDk1ODE5MDkxNzk2OSwid2lkdGgiOjEwLjAwODIzNDk3NzcyMjE2OCwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo4OC4zNjAwMzg3NTczMjQyMiwibGVmdCI6NTEuNDc3NjU3MzE4MTE1MjM0LCJ3aWR0aCI6MzguNjcyMDU0MjkwNzcxNDg0LCJoZWlnaHQiOjEuNDcxMTAxNTIyNDQ1Njc4N319LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjkwLjAwMzExMjc5Mjk2ODc1LCJsZWZ0Ijo1MS40Nzc2NTczMTgxMTUyMzQsIndpZHRoIjozOC42NzY0OTg0MTMwODU5NCwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo5MS42MzgyNTIyNTgzMDA3OCwibGVmdCI6NTEuNDc3NjUzNTAzNDE3OTcsIndpZHRoIjozOC42NzU4MzQ2NTU3NjE3MiwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjksImJib3giOnsidG9wIjo2LjQ2ODkzNTk2NjQ5MTY5OSwibGVmdCI6OS44NDAxMzM2NjY5OTIxODgsIndpZHRoIjozOC42ODYzMjUwNzMyNDIxOSwiaGVpZ2h0IjoxNi4yMTIxMjU3NzgxOTgyNDJ9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI5In19LHsib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhhbnMgVy4iLCJmYW1pbHkiOiJQYWVybCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLYXJsIEUuIiwiZmFtaWx5IjoiSGF2ZW5zIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hdGhhbiBTLiIsImZhbWlseSI6IkhhbGwifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiVGltb3RoeSBHLiIsImZhbWlseSI6Ik90dGVuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1lbmd5dWFuIiwiZmFtaWx5IjoiWmh1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhhaSIsImZhbWlseSI6Ilh1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikd1YW5nd2VpIiwiZmFtaWx5IjoiWmh1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkJvcWlhbmciLCJmYW1pbHkiOiJRaW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIxMzIzLTE2NTAiLCIxNDQ4LTYwNTkiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwNzEvbWYxODM5MiIsIm1hZyI6IjI5NDIzMjE1NDIiLCJvcGVuYWxleCI6IlcyOTQyMzIxNTQyIn0sInRpdGxlIjoiTWl0aWdhdGluZyBhIGdsb2JhbCBleHBhbnNpb24gb2YgdG94aWMgY3lhbm9iYWN0ZXJpYWwgYmxvb21zOiBjb25mb3VuZGluZyBlZmZlY3RzIGFuZCBjaGFsbGVuZ2VzIHBvc2VkIGJ5IGNsaW1hdGUgY2hhbmdlIiwiYWJzdHJhY3QiOiJNYW5hZ2luZyBhbmQgbWl0aWdhdGluZyB0aGUgZ2xvYmFsIGV4cGFuc2lvbiBvZiB0b3hpYyBjeWFub2JhY3RlcmlhbCBoYXJtZnVsIGFsZ2FsIGJsb29tcyAoQ3lhbm9IQUJzKSBpcyBhIG1ham9yIGNoYWxsZW5nZSBmYWNpbmcgcmVzZWFyY2hlcnMgYW5kIHdhdGVyIHJlc291cmNlIG1hbmFnZXJzLiBWYXJpb3VzIGFwcHJvYWNoZXMsIGluY2x1ZGluZyBudXRyaWVudCBsb2FkIHJlZHVjdGlvbiwgYXJ0aWZpY2lhbCBtaXhpbmcgYW5kIGZsdXNoaW5nLCBvbW5pdm9yb3VzIGZpc2ggcmVtb3ZhbCwgYWxnYWVjaWRlIGFwcGxpY2F0aW9ucyBhbmQgc2VkaW1lbnQgZHJlZGdpbmcsIGhhdmUgYmVlbiB1c2VkIHRvIHJlZHVjZSBibG9vbSBvY2N1cnJlbmNlcy4gSG93ZXZlciwgbWFuYWdlcnMgbm93IGZhY2UgdGhlIGFkZGl0aW9uYWwgY2hhbGxlbmdlIG9mIGhhdmluZyB0byBhZGRyZXNzIHRoZSBlZmZlY3RzIG9mIGNsaW1hdGUgY2hhbmdlIG9uIHdhdGVyc2hlZCBoeWRyb2xvZ2ljYWwgYW5kIG51dHJpZW50IGxvYWQgZHluYW1pY3MsIHdhdGVyIHRlbXBlcmF0dXJlLCBtaXhpbmcgcmVnaW1lIGFuZCBpbnRlcm5hbCBudXRyaWVudCBjeWNsaW5nLiBSaXNpbmcgdGVtcGVyYXR1cmVzIGFuZCBpbmNyZWFzaW5nIGZyZXF1ZW5jaWVzIGFuZCBtYWduaXR1ZGVzIG9mIGV4dHJlbWUgd2VhdGhlciBldmVudHMsIGluY2x1ZGluZyB0cm9waWNhbCBjeWNsb25lcywgZXh0cmF0cm9waWNhbCBzdG9ybXMsIGZsb29kcyBhbmQgZHJvdWdodHMsIGFsbCBwcm9tb3RlIEN5YW5vSEFCcyBhbmQgYWZmZWN0IHRoZSBlZmZpY2FjeSBvZiBlY29zeXN0ZW0gcmVtZWRpYXRpb24gbWVhc3VyZXMuIFRoZXNlIGNsaW1hdGljIGNoYW5nZXMgd2lsbCBsaWtlbHkgcmVxdWlyZSBzZXR0aW5nIHN0cmljdGVyIG51dHJpZW50IChpbmNsdWRpbmcgYm90aCBuaXRyb2dlbiBhbmQgcGhvc3Bob3J1cykgcmVkdWN0aW9uIHRhcmdldHMgZm9yIGJsb29tIGNvbnRyb2wgaW4gYWZmZWN0ZWQgd2F0ZXJzLiBJbiBhZGRpdGlvbiwgdGhlIGVmZmljYWN5IG9mIGN1cnJlbnRseSB1c2VkIG1ldGhvZHMgdG8gcmVkdWNlIEN5YW5vSEFCcyB3aWxsIG5lZWQgdG8gYmUgcmUtZXZhbHVhdGVkIGluIGxpZ2h0IG9mIHRoZSBzeW5lcmdpc3RpYyBlZmZlY3RzIG9mIGNsaW1hdGUgY2hhbmdlIHdpdGggbnV0cmllbnQgZW5yaWNobWVudC4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTA3MS9tZjE4MzkyIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2Nkci5saWIudW5jLmVkdS9kb3dubG9hZHMvMDU3NDIyODM0IiwiaXNzdWVkRGF0ZSI6IjIwMTktMDMtMjUiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTEwIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJNYXJpbmUgYW5kIEZyZXNod2F0ZXIgUmVzZWFyY2giLCJpc3N1ZSI6NSwidm9sdW1lIjo3MSwiaW1wYWN0RmFjdG9yIjoiMi43MCJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjU3OSwibGFzdFBhZ2UiOjU3OX19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJDU0lSTyBQdWJsaXNoaW5nIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjoxMDQsImNpdGVkQnlDb3VudCI6OTF9fSwicXVvdGUiOiJUaGF0IHJlbGF0aW9uc2hpcCBoYXMgc2VydmVkIGFzIHRoZSBmb3VuZGF0aW9uIGZvciBzZXR0aW5nICBudXRyaWVudCBsb2FkaW5nIHRhcmdldHMgaW4gbGFrZXMgYXJvdW5kIHRoZSB3b3JsZCwgYW5kIGNhcmVmdWxseSAgZGVzaWduZWQgZXhwZXJpbWVudHMgbWF5IGVsdWNpZGF0ZSB0byB3aGF0IGV4dGVudCB0ZW1wZXJhdHVyZSAgaW5jcmVhc2UgYWZmZWN0cyB0aGUgdGhyZXNob2xkIGF0IHdoaWNoIHRoZSBmcmVxdWVuY3kgb2Ygb2NjdXIgIHJlbmNlIG9mIEN5YW5vSEFCcyBxdWlja2x5IHJpc2VzIHdpdGggaW5jcmVhc2luZyBOIGFuZCBQLiBDb25jbHVzaW9ucyBSZWNlbnQgc3R1ZGllcyBpbmRpY2F0ZSB0aGF0IHRoZSBzeW5lcmdpc3RpYyBjb21iaW5hdGlvbiBvZiBhbnRocm9wb2dlbmljIG51dHJpZW50IGxvYWRpbmcsIHJpc2luZyB0ZW1wZXJhdHVyZXMsIGVuaGFuY2VkICB2ZXJ0aWNhbCBzdHJhdGlmaWNhdGlvbiwgaW5jcmVhc2VkIHJlc2lkZW5jZSB0aW1lIGFuZCBtb3JlICBleHRyZW1lIGNsaW1hdGljIGNvbmRpdGlvbnMgb3ZlcmFsbCB3aWxsIGZhdm91ciBjeWFub2JhY3RlcmlhbCAgZG9taW5hbmNlIGluIGEgd2lkZSByYW5nZSBvZiBhcXVhdGljIGVjb3N5c3RlbXMuIFRoZSBleHBhbiAgc2lvbiBvZiBDeWFub0hBQnMgaGFzIHNlcmlvdXMgY29uc2VxdWVuY2VzIGZvciBodW1hbiB3YXRlciAgc3VwcGxpZXMsIGZpc2hlcmllcyBhbmQgcmVjcmVhdGlvbmFsIHJlc291cmNlcy4gRWNvc3lzdGVtIHNjYWxlICBwaHlzaWNhbCBjaGVtaWNhbCBiaW90aWMgbWl0aWdhdGlvbiBzdHJhdGVnaWVzIHdpbGwgaGF2ZSB0byAgaW5jb3Jwb3JhdGUgbnV0cmllbnQgKG1vc3Qgb2Z0ZW4gYm90aCBOIGFuZCBQKSBpbnB1dCByZWR1Y3Rpb25zLiAgVGhlIG1hZ25pdHVkZXMgb2YgdGhlc2UgcmVkdWN0aW9ucyB3aWxsIG5lZWQgdG8gYmUgc3lzdGVtICBzcGVjaWZpYyBhbmQgd2lsbCBsaWtlbHkgbmVlZCB0byBiZSBhZGp1c3RlZCB3aXRoIGNoYW5naW5nIGNsaSAgbWF0aWMgY29uZGl0aW9ucy4gU3BlY2lmaWNhbGx5LCB3YXRlciBxdWFsaXR5IG1hbmFnZXJzIHdpbGwgaGF2ZSAgdG8gYWNjb21tb2RhdGUgd2FybWluZyBhbmQgZ3JlYXRlciBoeWRyb2xvZ2ljYWwgdmFyaWFiaWxpdHkgIGFuZCBleHRyZW1lbmVzcyByZXN1bHRpbmcgZnJvbSBjbGltYXRlIGNoYW5nZSBpbiBmb3JtdWxhdGluZyAgdGhlaXIgc3RyYXRlZ2llcy4gQSBrZXkgbG9uZyB0ZXJtIGNvbnRyb2wgd2UgY2FuIGV4ZXJ0IHRvIHJlZHVjZSAgdGhlIHJhdGUgYW5kIGV4dGVudCBvZiBnbG9iYWwgd2FybWluZyBpcyBjdXJiaW5nIGdyZWVuaG91c2UgZ2FzICBlbWlzc2lvbnMuIFdpdGhvdXQgdGhpcyBlc3NlbnRpYWwgc3RlcCwgZnV0dXJlIHdhcm1pbmcgdHJlbmRzLCAgaHlkcm9sb2dpY2FsIGV4dHJlbWVzIChyZWNvcmQgcmFpbmZhbGxzIGFuZCBkcm91Z2h0cykgYW5kIHRoZWlyICBlZmZlY3RzIG9uIGFxdWF0aWMgZWNvc3lzdGVtcyB3aWxsIGNvbnRpbnVlIHRvIHByb21vdGUgb3Bwb3IgIHR1bmlzdGljIEN5YW5vSEFCcy4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6MTQuMTQ5MjMzODE4MDU0MiwibGVmdCI6MTMuMTI4NTk1MzUyMTcyODUyLCJ3aWR0aCI6MzcuMDgyMTcyMzkzNzk4ODMsImhlaWdodCI6MS41MTY2MDIyNzc3NTU3MzczfX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjE1LjQ1MTUwODUyMjAzMzY5MSwibGVmdCI6MTMuMTI4NTQ0ODA3NDM0MDgyLCJ3aWR0aCI6MzcuMDg0ODMxMjM3NzkyOTcsImhlaWdodCI6MS41MTY2MDMyMzE0MzAwNTM3fX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjE2Ljc1Mzc4NDE3OTY4NzUsImxlZnQiOjEzLjEyODU0MjkwMDA4NTQ1LCJ3aWR0aCI6MzcuMDgzMjA5OTkxNDU1MDgsImhlaWdodCI6MS41MTY2MDQxODUxMDQzNzAxfX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjE4LjA1NjA1MzE2MTYyMTA5NCwibGVmdCI6MTMuMTI4NTQ0ODA3NDM0MDgyLCJ3aWR0aCI6MzYuNjE1MDc0MTU3NzE0ODQ0LCJoZWlnaHQiOjEuNTE2NjA0MTg1MTA0MzcwMX19LHsicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjoxOS40NjgzNzgwNjcwMTY2LCJsZWZ0IjoxMy4xMjg1NDQ4MDc0MzQwODIsIndpZHRoIjozNC4yOTA1MzExNTg0NDcyNjYsImhlaWdodCI6MS4yNDUwMTEyMTA0NDE1ODk0fX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjIyLjIyNjE0Mjg4MzMwMDc4LCJsZWZ0IjoxMy4xMjg1NTA1Mjk0Nzk5OCwid2lkdGgiOjcuNjQ5ODczNzMzNTIwNTA4LCJoZWlnaHQiOjEuMTM1NDgzMTQ1NzEzODA2Mn19LHsicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjoyMy43MzQ4NDk5Mjk4MDk1NywibGVmdCI6MTMuMTI4NTM1MjcwNjkwOTE4LCJ3aWR0aCI6MzcuMDg5Mjg2ODA0MTk5MjIsImhlaWdodCI6MjYuMDg4NjQwMjEzMDEyNjk1fX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNTg5In19XQ%3D%3D. Microcystins, the most widespread, inhibit protein phosphatases, disrupting cellular signaling and inducing oxidative stress, while neurotoxins target neuronal ion channels, causing rapid paralysis (Wang et al., 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ilhpd2VuIiwiZmFtaWx5IjoiV2FuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLdW4iLCJmYW1pbHkiOiJTaGkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiWXVubGluIiwiZmFtaWx5IjoiWmhhbmcifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQm9xaWFuZyIsImZhbWlseSI6IlFpbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJZaWJvIiwiZmFtaWx5IjoiWmhhbmcifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiV2VpamlhIiwiZmFtaWx5IjoiV2FuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJSLiBJZXN0eW4iLCJmYW1pbHkiOiJXb29sd2F5In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNoaWxvbmciLCJmYW1pbHkiOiJQaWFvIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkVyaWsiLCJmYW1pbHkiOiJKZXBwZXNlbiJ9fV0sImlkcyI6eyJpc3NuIjpbXSwiaXNibiI6W10sImRvaSI6IjEwLjIxMjAzL3JzLjMucnMtMjQ0MjE0Mi92MSIsIm9wZW5hbGV4IjoiVzQzMTcwMzg4MjcifSwidGl0bGUiOiJMYWtlIHdhcm1pbmcgYW5kIGluY3JlYXNlZCBoZWF0d2F2ZXMgcmV2ZWFsZWQgYnkgYSBub3ZlbCBkYXRhLWRyaXZlbiBtb2RlbGluZyBhcHByb2FjaCIsImFic3RyYWN0IjoiPHRpdGxlPkFic3RyYWN0PC90aXRsZT4gQ2xpbWF0ZSBjaGFuZ2UgY291bGQgc2VyaW91c2x5IHRocmVhdGVuIGdsb2JhbCBsYWtlIGVjb3N5c3RlbXMgYnkgd2FybWluZyBsYWtlIHN1cmZhY2Ugd2F0ZXIgYW5kIGluY3JlYXNpbmcgdGhlIG9jY3VycmVuY2Ugb2YgbGFrZSBoZWF0d2F2ZXMuIFlldCwgdGhlcmUgYXJlIGdyZWF0IHVuY2VydGFpbnRpZXMgaW4gcXVhbnRpZnlpbmcgbGFrZSB0ZW1wZXJhdHVyZSBjaGFuZ2VzIGdsb2JhbGx5IGR1ZSB0byBsYWNrIG9mIGFjY3VyYXRlIGxhcmdlLXNjYWxlIG1vZGVsIHNpbXVsYXRpb25zLiBIZXJlLCB3ZSBpbnRlZ3JhdGVkIHNhdGVsbGl0ZSBvYnNlcnZhdGlvbnMgYW5kIGEgbnVtZXJpY2FsIG1vZGVsIHRvIGltcHJvdmUgbGFrZSB0ZW1wZXJhdHVyZSBtb2RlbGxpbmcgYW5kIHRvIGV4cGxvcmUgdGhlIG11bHRpZmFjZXRlZCBjaGFyYWN0ZXJpc3RpY3Mgb2YgdHJlbmRzIGluIHN1cmZhY2UgdGVtcGVyYXR1cmVzIGFuZCBsYWtlIGhlYXR3YXZlIG9jY3VycmVuY2UgaW4gQ2hpbmVzZSBsYWtlcyBmcm9tIDE5ODAgdG8gMjEwMC4gT3VyIG1vZGVsLWRhdGEgaW50ZWdyYXRpb24gYXBwcm9hY2ggcmV2ZWFsZWQgdGhhdCB0aGUgbGFrZSBzdXJmYWNlIHdhdGVycyBoYXZlIHdhcm1lZCBhdCBhIHJhdGUgb2YgMC4xMSDCsEMgZGVjYWRlLTEgZHVyaW5nIHRoZSBwZXJpb2QgMTk4MC0yMDIxLCBiZWluZyBvbmx5IGhhbGYgb2YgdGhlIHB1cmUgbW9kZWwtYmFzZWQgZXN0aW1hdGUuIE1vcmVvdmVyLCBvdXIgYW5hbHlzaXMgc3VnZ2VzdGVkIHRoYXQgYW4gYXN5bW1ldHJpYyBzZWFzb25hbCB3YXJtaW5nIHJhdGUgaGFzIGxlZCB0byBhIHJlZHVjZWQgdGVtcGVyYXR1cmUgc2Vhc29uYWxpdHkgaW4gZWFzdGVybiBwbGFpbiBsYWtlcyBidXQgYW4gYW1wbGlmaWVkIG9uZSBpbiBhbHBpbmUgbGFrZXMuIFRoZSBkdXJhdGlvbnMgb2YgbGFrZSBoZWF0d2F2ZXMgaGF2ZSBhbHNvIGluY3JlYXNlZCBhdCBhIHJhdGUgb2YgNy43IGRheXMgZGVjYWRlLTEuIFVuZGVyIHRoZSBoaWdoLWdyZWVuaG91c2UtZ2FzLWVtaXNzaW9uIHNjZW5hcmlvLCBsYWtlIHN1cmZhY2UgdGVtcGVyYXR1cmUgYW5kIGxha2UgaGVhdHdhdmUgZHVyYXRpb24gd2VyZSBwcm9qZWN0ZWQgdG8gaW5jcmVhc2UgYnkgMi4yIMKwQyBhbmQgMTk3IGRheXMgYXQgdGhlIGVuZCBvZiAyMXN0IGNlbnR1cnksIHJlc3BlY3RpdmVseS4gU3VjaCBkcmFzdGljIGNoYW5nZXMgd291bGQgd29yc2VuIHRoZSBlbnZpcm9ubWVudGFsIGNvbmRpdGlvbnMgb2YgbGFrZXMgc3ViamVjdGVkIHRvIGhpZ2ggYW5kIGluY3JlYXNpbmcgYW50aHJvcG9nZW5pYyBwcmVzc3VyZXMsIHBvc2luZyBncmVhdCB0aHJlYXRzIHRvIGFxdWF0aWMgYmlvZGl2ZXJzaXR5IGFuZCBodW1hbiBoZWFsdGguIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjIxMjAzL3JzLjMucnMtMjQ0MjE0Mi92MSIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cucmVzZWFyY2hzcXVhcmUuY29tL2FydGljbGUvcnMtMjQ0MjE0Mi9sYXRlc3QucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjMtMDEtMTciLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTEwIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJSZXNlYXJjaCBTcXVhcmUgKFJlc2VhcmNoIFNxdWFyZSkiLCJpbXBhY3RGYWN0b3IiOiIwLjQwIn0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlJlc2VhcmNoIFNxdWFyZSAoVW5pdGVkIFN0YXRlcykifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjU3LCJjaXRlZEJ5Q291bnQiOjN9fSwicXVvdGUiOiJJZiB0aGUgYm90dG9tIHdhdGVyIHRlbXBlcmF0dXJlIGFsc28gaW5jcmVhc2VzLCBtaW5lcmFsaXphdGlvbiBvZiBvcmdhbmljIG1hdHRlciB3aWxsIGJlIGFjY2VsZXJhdGVkLCB3aGljaCB3aWxsIGxlYWQgdG8gaGlnaGVyIGVtaXNzaW9uIG9mIGNhcmJvbiBkaW94aWRlIGFuZCBtZXRoYW5lNDYsNDcsIHJlaW5mb3JjaW5nIGNsaW1hdGUgd2FybWluZyBkdWUgdG8gZ3JlZW5ob3VzZSBnYXNlcy4gVGhpcyBwcm9jZXNzIHdpbGwgc3BlZWQgdXAgdGhlIHJlY3ljbGluZyBvZiBudXRyaWVudHMgYW5kIGZhdm9yIHBoeXRvcGxhbmt0b24gZ3Jvd3RoLCBzdWdnZXN0aW5nIHBvc2l0aXZlIGZlZWRiYWNrIGJldHdlZW4gY2xpbWF0ZSBjaGFuZ2UgYW5kIGV1dHJvcGhpY2F0aW9uNDguIFRvZ2V0aGVyIHdpdGggdGhlIGhlYXZ5IGV4dGVybmFsIG51dHJpZW50IGxvYWRpbmcgZnJvbSBkb21lc3RpYywgYWdyaWN1bHR1cmFsLCBhbmQgaW5kdXN0cmlhbCB3YXN0ZSwgdGhlIGdyb3d0aCBvZiBwaHl0b3BsYW5rdG9uIGlzIHByb21vdGVkIGJ5IGhvdCB3YXRlciB1bmRlciBudXRyaWVudC1yaWNoIGNvbmRpdGlvbnM0OS4gQmVzaWRlcyB0aGF0LCBzdHJvbmdlciBzdHJhdGnvv71jYXRpb24gd2lsbCBwZXJtaXQgcGh5dG9wbGFua3RvbiB0byBjaXJjdWxhdGUgaW4gdGhlIGV1cGhvdGljIHpvbmU1MC4gSW5jcmVhc2luZyBhbGdhZSBibG9vbXMgaGF2ZSBiZWVuIHJlcG9ydGVkIHdvcmxkd2lkZSwgZXNwZWNpYWxseSBpbiByZWdpb25zIHRoYXQgZGVwZW5kIGhpZ2hseSBvbiBmZXJ0aWxpemVyczUxLiBJbiBDaGluYSwgdGhlIG9jY3VycmVuY2Ugb2YgYWxnYWUgYmxvb21zIGluIHRoZSAyMDEwcyBoYXMgaW5jcmVhc2VkIG5lYXJseSBuaW5lIHRpbWVzIHJlbGF0aXZlIHRvIHRoZSAxOTgwcy0xOTkwczUxLCBhbmQgbWFueSBibG9vbXMgaGF2ZSBiZWVuIHJlcG9ydGVkIHJlY2VudGx5IGluIG5hdHVyYWwgbGFrZXMgYW5kIHJlc2Vydm9pcnMgKFN1cHBsZW1lbnRhcnkgRmlnLiA3KS4gRnVydGhlcm1vcmUsIHRoZSB3YXJtZXIgc3VyZmFjZSB3YXRlciBhbmQgc3Ryb25nZXIgc3RyYXRp77%2B9Y2F0aW9uIHdpbGwgZmF2b3IgdGhlIGRvbWluYW5jZSBvZiBjeWFub2JhY3RlcmlhIGluIGxha2VzIGdpdmVuIGl0cyBoaWdoZXIgb3B0aW1hbCBncm93dGggdGVtcGVyYXR1cmUgYW5kIGFiaWxpdHkgdG8gYWRqdXN0IGJ1b3lhbmN5NDksIGxlYWRpbmcgdG8gaGlnaGVyIGZyZXF1ZW5jeSBvZiBoYXJtZnVsIGFsZ2FlIGJsb29tczQ3LiBUaGUgdG94aWMgc3Vic3RhbmNlcyBwcm9kdWNlZCBieSBjeWFub2JhY3RlcmlhIHdpbGwgdGhyZWF0ZW4gdGhlIHNlY3VyaXR5IG9mIGRyaW5raW5nIHdhdGVyIGFuZCBodW1hbiBoZWFsdGg2LDUyLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjQ1LjEyNjk2MDc1NDM5NDUzLCJsZWZ0Ijo0LjU3NTE2Mjg4NzU3MzI0Miwid2lkdGgiOjkwLjk0MTIwMDI1NjM0NzY2LCJoZWlnaHQiOjEuNzc3MTYzMzg2MzQ0OTA5N319LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjQ3LjQwMTcyMTk1NDM0NTcsImxlZnQiOjQuNTc1MTYyODg3NTczMjQyLCJ3aWR0aCI6OTAuMjA1MjQ1OTcxNjc5NjksImhlaWdodCI6MS43NzcxNjMzODYzNDQ5MDk3fX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NDkuNjQ5MTE2NTE2MTEzMjgsImxlZnQiOjQuNTc1MTYyODg3NTczMjQyLCJ3aWR0aCI6OTEuMDQzNzMxNjg5NDUzMTIsImhlaWdodCI6NC4yNjg4Njk4NzY4NjE1NzJ9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo1NC4zODgyMTc5MjYwMjUzOSwibGVmdCI6NC41NzUxNjI4ODc1NzMyNDIsIndpZHRoIjo0OC40NzYyMjY4MDY2NDA2MjUsImhlaWdodCI6MS45OTQxMDgwODA4NjM5NTI2fX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NTguMzk2NDM0NzgzOTM1NTUsImxlZnQiOjQuNTc1MTYyODg3NTczMjQyLCJ3aWR0aCI6ODcuODgyMzg1MjUzOTA2MjUsImhlaWdodCI6MS43NzcxNjMzODYzNDQ5MDk3fX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NjAuNjQzODI5MzQ1NzAzMTI1LCJsZWZ0Ijo0LjU3NTE2Mjg4NzU3MzI0Miwid2lkdGgiOjkwLjQ1MDU2MTUyMzQzNzUsImhlaWdodCI6MS45OTQxMDgwODA4NjM5NTI2fX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NjMuMTA4MTY5NTU1NjY0MDYsImxlZnQiOjQuNTc1MTYyODg3NTczMjQyLCJ3aWR0aCI6ODkuOTY3NTk3OTYxNDI1NzgsImhlaWdodCI6MS45OTQxMDgwODA4NjM5NTI2fX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NjUuNTcyNDQ4NzMwNDY4NzUsImxlZnQiOjQuNTc1MTYyODg3NTczMjQyLCJ3aWR0aCI6OTAuNTgwODg2ODQwODIwMzEsImhlaWdodCI6MS45OTQxNjk3MTIwNjY2NTA0fX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NjguMDM2Nzg4OTQwNDI5NjksImxlZnQiOjQuNTc1MTYyODg3NTczMjQyLCJ3aWR0aCI6ODkuMTkwNDM3MzE2ODk0NTMsImhlaWdodCI6Ni41NDM2OTMwNjU2NDMzMTA1fX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NzUuMDUwNjUxNTUwMjkyOTcsImxlZnQiOjQuNTc1MTYyODg3NTczMjQyLCJ3aWR0aCI6OTAuMzQ3MDY4Nzg2NjIxMSwiaGVpZ2h0IjoxLjk5NDE2OTcxMjA2NjY1MDR9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo3Ny41MTQ5OTE3NjAyNTM5LCJsZWZ0Ijo0LjU3NTE2Mjg4NzU3MzI0Miwid2lkdGgiOjkwLjIyMjQ4ODQwMzMyMDMxLCJoZWlnaHQiOjEuOTk0MTY5NzEyMDY2NjUwNH19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjc5Ljk3OTMzOTU5OTYwOTM4LCJsZWZ0Ijo0LjU3NTE2Mjg4NzU3MzI0Miwid2lkdGgiOjg1Ljk4MjEzMTk1ODAwNzgxLCJoZWlnaHQiOjEuOTk0MTY5NzEyMDY2NjUwNH19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjgifX1d. In polymictic lakes such as Bellandur and Hussainsagar, persistent surface scums concentrate these toxins at levels exceeding WHO guidelines (1 μg/L for microcystin-LR in drinking water), amplifying risks during peak blooms (Shahnazaryan et al., 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkdheWFuZSIsImZhbWlseSI6IlNoYWhuYXphcnlhbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNYXJ0aW4iLCJmYW1pbHkiOiJTY2h1bHR6ZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLYXJzdGVuIiwiZmFtaWx5IjoiUmlua2UifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQmFyZHVraCIsImZhbWlseSI6IkdhYnJpZWx5YW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIxMTI5LTU3NjciLCIxNzIzLTg2MzMiXSwiaXNibiI6W10sImRvaSI6IjEwLjQwODEvamxpbW5vbC4yMDIyLjIxNjgiLCJvcGVuYWxleCI6Ilc0Mzg4OTc3MDMyIn0sInRpdGxlIjoiTGFrZSBTZXZhbi4gUGFzdCwgcHJlc2VudCwgYW5kIGZ1dHVyZSBzdGF0ZSBvZiBhIHVuaXF1ZSBhbHBpbmUgbGFrZSIsImFic3RyYWN0IjoiTGFrZSBTZXZhbiwgYSBsYXJnZSwgZGVlcCwgYWxwaW5lIGxha2UgaW4gdGhlIExlc3NlciBDYXVjYXN1cyBpcyB0aGUgZm9jdXMgb2YgdGhpcyBTcGVjaWFsIElzc3VlIG9mIHRoZSBKb3VybmFsIG9mIExpbW5vbG9neS4gSXQgd2FzIGFuIG91dHN0YW5kaW5nIGVjb3N5c3RlbSAxMDAgeWVhcnMgYWdvIGNoYXJhY3RlcmlzZWQgYnkgZXhjZWxsZW50IHdhdGVyIHF1YWxpdHksIHJpY2ggYmlvZGl2ZXJzaXR5IHdpdGggYSBoaWdoIGxldmVsIG9mIGVuZGVtaXNtLCB3aWRlLXJhbmdpbmcgYmVkcyBvZiBtYWNyb3BoeXRlcyBhbG9uZyB0aGUgc2hvcmVzIGFuZCBhIHByb2R1Y3RpdmUgYW5kIHN1c3RhaW5hYmxlIGZpc2ggcHJvZHVjdGlvbi4gRHVlIHRvIGl0cyBiZWF1dHksIG5hdHVyYWwgaGlzdG9yeSwgYW5kIGNvbnRyaWJ1dGlvbnMgdG8gc29jaWFsIGFuZCBlY29ub21pYyB3ZWxmYXJlIGl0IGlzIGFsc28gYSBjdWx0dXJhbCBoZXJpdGFnZSBmb3IgdGhlIEFybWVuaWFuIE5hdGlvbiBpbmNsdWRpbmcgaXRzIGxhcmdlIGRpYXNwb3JhLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC40MDgxL2psaW1ub2wuMjAyMi4yMTY4IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5qbGltbm9sLml0L2luZGV4LnBocC9qbGltbm9sL2FydGljbGUvZG93bmxvYWQvMjE2OC8yMDA3IiwiaXNzdWVkRGF0ZSI6IjIwMjMtMTEtMjMiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA5IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJKb3VybmFsIG9mIExpbW5vbG9neSIsInZvbHVtZSI6ODEsImltcGFjdEZhY3RvciI6IjIuMDEifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiUEFHRVByZXNzIChJdGFseSkifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjM1LCJjaXRlZEJ5Q291bnQiOjJ9fSwicXVvdGUiOiJDbGltYXRlIHdhcm1pbmcgY2F1c2VzIGNoYW5nZXMgaW4gdGhlIHRoZXJtYWwgcmVnaW1lIG9mIGxha2VzLCBpLmUuLCB0aGUgdHlwaWNhbCB0ZW1wZXJhdHVyZSBzdHJ1Y3R1cmVzIChXb29sd2F5IGFuZCBNZXItY2hhbnQsIDIwMTkpIGFuZCB0aGUgdGltaW5nIG9mIG1peGluZyBhbmQgc3RyYXRpZmljYXRpb24gKEtyYWVtZXIgZXQgYWwuLCAyMDIxKSwgdGhlIGR1cmF0aW9uIG9mIHN1bW1lciBzdHJhdGlmaWNhdGlvbiAgYW5kIGljZSBjb3ZlciAoUm9nb3JhIGV0IGFsLiwgMjAxODsgU2hhcm1hIGV0IGFsLiwgMjAxOSkgYW5kLCBldmVudHVhbGx5LCBpdCBpcyBhY2NlbGVyYXRpbmcgaW50ZXJuYWwgbnV0cmllbnQgbG9hZGluZyAoTm9ydGggZXQgYWwuLCAyMDE0KSBhbmQgaW1wYWN0aW5nIHRoZSBlbnRpcmUgbGFrZSBlY29zeXN0ZW0gIChTY2h3ZWZlbCBldCBhbC4sIDIwMTk7IFdvb2x3YXkgZXQgYWwuLCAyMDIyKS4gUmVjZW50IHN0dWRpZXMgd29ybGR3aWRlIGRvY3VtZW50ZWQgdW5lcXVpdm9jYWxseSB0aGF0IHRoZSB0eXBpY2FsIHN0cmVzc29ycyBvZiBsYWtlcyBpbnRlcmFjdCwgcG90ZW50aWFsbHkgbGVhZC1pbmcgdG8gZW5mb3JjZWQgbmVnYXRpdmUgY29uc2VxdWVuY2VzLiBUaGVyZSBpcyBjb25zaWRlci1hYmxlIGV2aWRlbmNlIHRoYXQgY2xpbWF0ZSBjaGFuZ2UgaXMgZXhhY2VyYmF0aW5nICBldXRyb3BoaWNhdGlvbiBhbmQgSEFCcyAoUGVhcmwgYW5kIEh1aXNtYW4sIDIwMDg7IFdvb2wtd2F5IGV0IGFsLiwgMjAyMDsgTWVlcmhvZmYgZXQgYWwuLCAyMDIyKSBhbmQgb3RoZXIgZWZmZWN0cyAga25vd24gZnJvbSBldXRyb3BoaWNhdGlvbiBsaWtlIGh5cG94aWEgYW5kIGFub3hpYSBvZiB0aGUgIGh5cG9saW1uaW9uIG9mIHN0cmF0aWZpZWQgbGFrZXMgKEZ1a3VzaGltYSBldCBhbC4sIDIwMTc7ICBSb2dvcmEgZXQgYWwuLCAyMDE4OyBKYW5lIGV0IGFsLiwgMjAyMSkuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NTQuMjUwMjMyNjk2NTMzMiwibGVmdCI6Ny4xNDMzOTMwMzk3MDMzNjksIndpZHRoIjozOC40OTkyNjc1NzgxMjUsImhlaWdodCI6MS4zOTUyNjMzMTQyNDcxMzEzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6MjUuNTIyNzkyODE2MTYyMTEsImxlZnQiOjQ5Ljk5OTE0NTUwNzgxMjUsIndpZHRoIjozOC41MDYyOTQyNTA0ODgyOCwiaGVpZ2h0IjoxMi4xMjk3NzUwNDczMDIyNDZ9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjozOS4zMjI2ODUyNDE2OTkyMiwibGVmdCI6NTIuMzgxMjM3MDMwMDI5Mywid2lkdGgiOjM2LjEyNDQ5NjQ1OTk2MDk0LCJoZWlnaHQiOjEuMzk1MjYzMzE0MjQ3MTMxM319LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjQwLjg1NjAxNDI1MTcwODk4NCwibGVmdCI6NTAuMDAwODIzOTc0NjA5Mzc1LCJ3aWR0aCI6MzguMDg0NDk1NTQ0NDMzNTk0LCJoZWlnaHQiOjEuMzk1MjYzMzE0MjQ3MTMxM319LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjQyLjM4OTMzNTYzMjMyNDIyLCJsZWZ0Ijo1MC4wMDA4MjM5NzQ2MDkzNzUsIndpZHRoIjozOC4wNDg1NTcyODE0OTQxNCwiaGVpZ2h0IjoxLjM5NTI2MzMxNDI0NzEzMTN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo0My45MjI2NjA4Mjc2MzY3MiwibGVmdCI6NTAuMDAwODIzOTc0NjA5Mzc1LCJ3aWR0aCI6MzguNTA1NTEyMjM3NTQ4ODMsImhlaWdodCI6MS4zOTUyNjMzMTQyNDcxMzEzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NDUuNDU1OTg2MDIyOTQ5MjIsImxlZnQiOjUwLjAwMDgyMzk3NDYwOTM3NSwid2lkdGgiOjM4LjA4MDc4MDAyOTI5Njg3NSwiaGVpZ2h0IjoxLjM5NTI2MzMxNDI0NzEzMTN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo0Ni45OTA1NjYyNTM2NjIxMSwibGVmdCI6NTAuMDAwODIzOTc0NjA5Mzc1LCJ3aWR0aCI6MzguNDk5OTQ2NTk0MjM4MjgsImhlaWdodCI6MS4zOTUyNjMzMTQyNDcxMzEzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NDguNTIyNjI4Nzg0MTc5NjksImxlZnQiOjUwLjAwMDgyMzk3NDYwOTM3NSwid2lkdGgiOjM4LjUwMzExMjc5Mjk2ODc1LCJoZWlnaHQiOjEuMzk1MjYzMzE0MjQ3MTMxM319LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjUwLjA1NzIxMjgyOTU4OTg0NCwibGVmdCI6NTAuMDAwODIzOTc0NjA5Mzc1LCJ3aWR0aCI6MzguNDk4OTM5NTE0MTYwMTU2LCJoZWlnaHQiOjEuMzk1MjYzMzE0MjQ3MTMxM319LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjUxLjU5MDUzODAyNDkwMjM0NCwibGVmdCI6NTAuMDAwODI3Nzg5MzA2NjQsIndpZHRoIjozOC41MDEwNzk1NTkzMjYxNywiaGVpZ2h0IjoxLjM5NTI2MzMxNDI0NzEzMTN9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIxIn19XQ%3D%3D. Climate-driven extensions of stratification and warmer temperatures further select for toxigenic strains, intensifying toxin production and release during cell lysis (Meerhoff et al., 2022)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik0uIiwiZmFtaWx5IjoiTWVlcmhvZmYifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSi4iLCJmYW1pbHkiOiJBdWRldCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJULkEuIiwiZmFtaWx5IjoiRGF2aWRzb24ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTC4gRGUiLCJmYW1pbHkiOiJNZWVzdGVyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlMuIiwiZmFtaWx5IjoiSGlsdCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTLiIsImZhbWlseSI6Iktvc3RlbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJaYWlodWEiLCJmYW1pbHkiOiJMaXUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTi4iLCJmYW1pbHkiOiJNYXp6ZW8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSGFucyBXLiIsImZhbWlseSI6IlBhZXJsIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik0uIiwiZmFtaWx5IjoiU2NoZWZmZXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiRS4iLCJmYW1pbHkiOiJKZXBwZXNlbiJ9fV0sImlkcyI6eyJpc3NuIjpbXSwiaXNibiI6W10sImRvaSI6IjEwLjE3NjE1Lzd2ZTUtdzAxNiIsIm9wZW5hbGV4IjoiVzQzMTg0MTU1NzcifSwidGl0bGUiOiJGZWVkYmFjayBiZXR3ZWVuIGNsaW1hdGUgY2hhbmdlIGFuZCBldXRyb3BoaWNhdGlvbjogcmV2aXNpdGluZyB0aGUgYWxsaWVkIGF0dGFjayBjb25jZXB0IGFuZCBob3cgdG8gc3RyaWtlIGJhY2siLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTc2MTUvN3ZlNS13MDE2IiwicmVzb3VyY2VVcmwiOiJodHRwczovL2Nkci5saWIudW5jLmVkdS9kb3dubG9hZHMvaDk4OXJjOTM4IiwiaXNzdWVkRGF0ZSI6IjIwMjItMDEtMDEiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTEwIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJDYXJvbGluYSBEaWdpdGFsIFJlcG9zaXRvcnkgKFVuaXZlcnNpdHkgb2YgTm9ydGggQ2Fyb2xpbmEgYXQgQ2hhcGVsIEhpbGwpIiwiaW1wYWN0RmFjdG9yIjoiMC4xMCJ9LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJVbml2ZXJzaXR5IG9mIE5vcnRoIENhcm9saW5hIGF0IENoYXBlbCBIaWxsIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjowLCJjaXRlZEJ5Q291bnQiOjN9fSwicXVvdGUiOiJXaXRoIGNsaW1hdGUgd2FybWluZywgdGhlIGRpcmVjdCBhbmQgaW5kaS1yZWN0IGVmZmVjdHMgb2YgaW5jcmVhc2VkIGxha2UgdGVtcGVyYXR1cmVzIHdpbGwgbGlrZWx5IGRyaXZlIGNoYW5nZXMgaW4gY3lhbm9iYWN0ZXJpYWwgdGF4YSBhbmQgdGhlaXIgcHJvZHVjLXRpb24gb2YgY3lhbm9iYWN0ZXJpYWwgdG94aW5zLCBsaWtlbHkgaW5jbHVkaW5nIHByb21vdC1pbmcgdGhlIHNlbGVjdGlvbiBvZiBhIGZldyBNaWNyb2N5c3RpcyBzcGVjaWVzIG9yIHN0cmFpbnMgcHJvZHVjaW5nIGhpZ2hseSB0b3hpYyBtaWNyb2N5c3Rpbi1MUiAoTWFudHpvdWtpIGV0IGFsLiAyMDE4KS4gRGVzcGl0ZSB2YXJpYXRpb25zIGFtb25nIGxha2VzIGluIGEgd2F0ZXItc2NhcGUsIGEgcmVnaW9uYWwgaW5jcmVhc2UgaW4gbWljcm9jeXN0aW4gbWF4aW1hIGFuZCBkdXJhdGlvbiBvZiBibG9vbXMgaGFzIGFsc28gYmVlbiBjbGVhcmx5IGV4cGxhaW5lZCBieSBpbmNyZWFzaW5nIHRlbXBlcmF0dXJlIGluIGNlbnRyYWwgTm9ydGggQW1lcmljYSAoSGF5ZXMgZXQgYWwuIDIwMjApLCB3aXRoIGluY3JlYXNpbmcgcmlza3MgdG8gaHVtYW4gaGVhbHRoLiBHcm93aW5nIGV2aWRlbmNlIGluZGljYXRlcyB0aGF0IGN5YW5vYmFjdGVyaWFsIGRvbWluYW5jZSB3aWxsIGV4ZXJ0IGEgcG9zaXRpdmUgZmVlZGJhY2sgd2l0aCBjbGltYXRlIHdhcm1pbmcgKEhhdmVucyBldCBhbC4gMjAxNiwgUGFlcmwgZXQgYWwuIDIwMjApLiBNYW55IGN5YW5vYmFjdGVyaWFsIHNwZWNpZXMgcHJvbGlmZXJhdGUgb24gdGhlIGxha2Ugc3VyZmFjZSwgYWJzb3JiaW5nIGxpZ2h0LCBlbmhhbmNpbmcgbG9jYWxpemVkIHRlbXBlcmF0dXJlIGluY3JlYXNlcywgYW5kIGluY3JlYXNpbmcgdGhlcm1hbCBzdHJhdGlmaWNhdGlvbiBwb3RlbnRpYWwgKEthaHJ1IGV0IGFsLiAxOTkzKS4gQ3lhbm9iYWN0ZXJpYSBwcm9tb3RlIHRoZSByZWxlYXNlIG9mIEdIR3MsIHBhcnRpY3VsYXJseSBDSDQgKFlhbiBldCBhbC4gMjAxNyksIGR1cmluZyB0aGUgZGVjb21wb3NpdGlvbiBmb2xsb3dpbmcgdGhlIGNvbGxhcHNlIG9mIGEgYmxvb20uIEN5YW5vYmFjdGVyaWEgYWxzbyBwcm9tb3RlIHJlbWluZXJhbGl6YXRpb24gb2YgbnV0cmktZW50cyBkdXJpbmcgYmxvb21zLCBsZWFkaW5nIHRvIG11bHRpcGxlIGJsb29tcyBpbiBhIHNpbmdsZSBzZWFzb24uIFVuZGVyIGV1dHJvcGhpYyBhbmQgd2FybWluZyBjb25kaS10aW9ucywgY3lhbm9iYWN0ZXJpYSBkb21pbmFuY2UgbWF5IHRodXMgYmUgYSBzZWxmLXBlcnBldHVhdGluZyBwcm9waGVjeSAoRmlnLiAyIGFuZCAzKS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo2Ny4wNjM0MDAyNjg1NTQ2OSwibGVmdCI6NTEuNDc4ODAxNzI3Mjk0OTIsIndpZHRoIjozOC42NzM0MjM3NjcwODk4NDQsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6NjguNjk4NTQ3MzYzMjgxMjUsImxlZnQiOjUxLjQ3ODgwMTcyNzI5NDkyLCJ3aWR0aCI6MzguNjc5NDk2NzY1MTM2NzIsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6NzAuMzMzNjg2ODI4NjEzMjgsImxlZnQiOjUxLjQ3ODgwMTcyNzI5NDkyLCJ3aWR0aCI6MzguNjc3NTgxNzg3MTA5Mzc1LCJoZWlnaHQiOjEuNDcxMTAxNTIyNDQ1Njc4N319LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjcxLjk2ODgyNjI5Mzk0NTMxLCJsZWZ0Ijo1MS40Nzg4MDE3MjcyOTQ5Miwid2lkdGgiOjM4LjY3OTEwMzg1MTMxODM2LCJoZWlnaHQiOjEuNDcxMTAxNTIyNDQ1Njc4N319LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjczLjYxMDU3MjgxNDk0MTQsImxlZnQiOjUxLjQ3ODgwMTcyNzI5NDkyLCJ3aWR0aCI6MzguNjc0NDI3MDMyNDcwNywiaGVpZ2h0IjoxLjQ3Mzc0Njg5NTc5MDF9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo3NS4yNDcwMzk3OTQ5MjE4OCwibGVmdCI6NTEuNDc4ODA5MzU2Njg5NDUsIndpZHRoIjozOC42NzQyNjMwMDA0ODgyOCwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo3Ni44ODIxODY4ODk2NDg0NCwibGVmdCI6NTEuNDc4ODA5MzU2Njg5NDUsIndpZHRoIjozOC42ODExNzE0MTcyMzYzMywiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo3OC41MjUyNTMyOTU4OTg0NCwibGVmdCI6NTEuNDc4ODA5MzU2Njg5NDUsIndpZHRoIjozOC42NzU2NjI5OTQzODQ3NjYsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODAuMTYwNDAwMzkwNjI1LCJsZWZ0Ijo1MS40Nzg4MDkzNTY2ODk0NSwid2lkdGgiOjM4LjY4MjAyOTcyNDEyMTA5NCwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo4MS43OTU1Mzk4NTU5NTcwMywibGVmdCI6NTEuNDc4ODA5MzU2Njg5NDUsIndpZHRoIjozOC42NzQ5ODM5NzgyNzE0ODQsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODMuNDM4NjEzODkxNjAxNTYsImxlZnQiOjUxLjQ3ODgwOTM1NjY4OTQ1LCJ3aWR0aCI6MzguNjc2MTg1NjA3OTEwMTU2LCJoZWlnaHQiOjEuNDg2OTczNTI0MDkzNjI4fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODUuMDg5NjMwMTI2OTUzMTIsImxlZnQiOjUxLjQ3ODgwOTM1NjY4OTQ1LCJ3aWR0aCI6NC42ODI4MDUwNjEzNDAzMzIsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODYuNzI0Nzc3MjIxNjc5NjksImxlZnQiOjUzLjQ0MTE3NzM2ODE2NDA2LCJ3aWR0aCI6Ni4xNDAzNTU1ODcwMDU2MTUsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODYuNzI0ODkxNjYyNTk3NjYsImxlZnQiOjYwLjk5NDg3Njg2MTU3MjI2Niwid2lkdGgiOjYuMDM2OTkyNTQ5ODk2MjQsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODYuNzI0ODkxNjYyNTk3NjYsImxlZnQiOjY4LjQ0NjUyNTU3MzczMDQ3LCJ3aWR0aCI6Ni4xNTA3MDU4MTQzNjE1NzIsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODYuNzI0ODkxNjYyNTk3NjYsImxlZnQiOjc2LjAwOTgyNjY2MDE1NjI1LCJ3aWR0aCI6Mi43MjczMzMzMDcyNjYyMzU0LCJoZWlnaHQiOjEuNDcxMTAxNTIyNDQ1Njc4N319LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjg2LjcyNDg5MTY2MjU5NzY2LCJsZWZ0Ijo4MC4xNDk1ODE5MDkxNzk2OSwid2lkdGgiOjEwLjAwODIzNDk3NzcyMjE2OCwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo4OC4zNjAwMzg3NTczMjQyMiwibGVmdCI6NTEuNDc3NjU3MzE4MTE1MjM0LCJ3aWR0aCI6MzguNjcyMDU0MjkwNzcxNDg0LCJoZWlnaHQiOjEuNDcxMTAxNTIyNDQ1Njc4N319LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjkwLjAwMzExMjc5Mjk2ODc1LCJsZWZ0Ijo1MS40Nzc2NTczMTgxMTUyMzQsIndpZHRoIjozOC42NzY0OTg0MTMwODU5NCwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo5MS42MzgyNTIyNTgzMDA3OCwibGVmdCI6NTEuNDc3NjUzNTAzNDE3OTcsIndpZHRoIjozOC42NzU4MzQ2NTU3NjE3MiwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjksImJib3giOnsidG9wIjo2LjQ2ODkzNTk2NjQ5MTY5OSwibGVmdCI6OS44NDAxMzM2NjY5OTIxODgsIndpZHRoIjozOC42ODYzMjUwNzMyNDIxOSwiaGVpZ2h0IjoxNi4yMTIxMjU3NzgxOTgyNDJ9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI5In19XQ%3D%3D.

Primary exposure pathways for humans and livestock include ingestion of contaminated drinking water, direct dermal or inhalation contact during recreation, and consumption of tainted fish or irrigated crops. In peri-urban Indian lakes, untreated sewage inflows elevate toxin bioavailability, with microcystins detected in raw water supplies at concentrations up to 10-50 μg/L, persisting through conventional chlorination due to their heat stability and peptide structure (Wang et al., 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ilhpd2VuIiwiZmFtaWx5IjoiV2FuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLdW4iLCJmYW1pbHkiOiJTaGkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiWXVubGluIiwiZmFtaWx5IjoiWmhhbmcifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQm9xaWFuZyIsImZhbWlseSI6IlFpbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJZaWJvIiwiZmFtaWx5IjoiWmhhbmcifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiV2VpamlhIiwiZmFtaWx5IjoiV2FuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJSLiBJZXN0eW4iLCJmYW1pbHkiOiJXb29sd2F5In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNoaWxvbmciLCJmYW1pbHkiOiJQaWFvIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkVyaWsiLCJmYW1pbHkiOiJKZXBwZXNlbiJ9fV0sImlkcyI6eyJpc3NuIjpbXSwiaXNibiI6W10sImRvaSI6IjEwLjIxMjAzL3JzLjMucnMtMjQ0MjE0Mi92MSIsIm9wZW5hbGV4IjoiVzQzMTcwMzg4MjcifSwidGl0bGUiOiJMYWtlIHdhcm1pbmcgYW5kIGluY3JlYXNlZCBoZWF0d2F2ZXMgcmV2ZWFsZWQgYnkgYSBub3ZlbCBkYXRhLWRyaXZlbiBtb2RlbGluZyBhcHByb2FjaCIsImFic3RyYWN0IjoiPHRpdGxlPkFic3RyYWN0PC90aXRsZT4gQ2xpbWF0ZSBjaGFuZ2UgY291bGQgc2VyaW91c2x5IHRocmVhdGVuIGdsb2JhbCBsYWtlIGVjb3N5c3RlbXMgYnkgd2FybWluZyBsYWtlIHN1cmZhY2Ugd2F0ZXIgYW5kIGluY3JlYXNpbmcgdGhlIG9jY3VycmVuY2Ugb2YgbGFrZSBoZWF0d2F2ZXMuIFlldCwgdGhlcmUgYXJlIGdyZWF0IHVuY2VydGFpbnRpZXMgaW4gcXVhbnRpZnlpbmcgbGFrZSB0ZW1wZXJhdHVyZSBjaGFuZ2VzIGdsb2JhbGx5IGR1ZSB0byBsYWNrIG9mIGFjY3VyYXRlIGxhcmdlLXNjYWxlIG1vZGVsIHNpbXVsYXRpb25zLiBIZXJlLCB3ZSBpbnRlZ3JhdGVkIHNhdGVsbGl0ZSBvYnNlcnZhdGlvbnMgYW5kIGEgbnVtZXJpY2FsIG1vZGVsIHRvIGltcHJvdmUgbGFrZSB0ZW1wZXJhdHVyZSBtb2RlbGxpbmcgYW5kIHRvIGV4cGxvcmUgdGhlIG11bHRpZmFjZXRlZCBjaGFyYWN0ZXJpc3RpY3Mgb2YgdHJlbmRzIGluIHN1cmZhY2UgdGVtcGVyYXR1cmVzIGFuZCBsYWtlIGhlYXR3YXZlIG9jY3VycmVuY2UgaW4gQ2hpbmVzZSBsYWtlcyBmcm9tIDE5ODAgdG8gMjEwMC4gT3VyIG1vZGVsLWRhdGEgaW50ZWdyYXRpb24gYXBwcm9hY2ggcmV2ZWFsZWQgdGhhdCB0aGUgbGFrZSBzdXJmYWNlIHdhdGVycyBoYXZlIHdhcm1lZCBhdCBhIHJhdGUgb2YgMC4xMSDCsEMgZGVjYWRlLTEgZHVyaW5nIHRoZSBwZXJpb2QgMTk4MC0yMDIxLCBiZWluZyBvbmx5IGhhbGYgb2YgdGhlIHB1cmUgbW9kZWwtYmFzZWQgZXN0aW1hdGUuIE1vcmVvdmVyLCBvdXIgYW5hbHlzaXMgc3VnZ2VzdGVkIHRoYXQgYW4gYXN5bW1ldHJpYyBzZWFzb25hbCB3YXJtaW5nIHJhdGUgaGFzIGxlZCB0byBhIHJlZHVjZWQgdGVtcGVyYXR1cmUgc2Vhc29uYWxpdHkgaW4gZWFzdGVybiBwbGFpbiBsYWtlcyBidXQgYW4gYW1wbGlmaWVkIG9uZSBpbiBhbHBpbmUgbGFrZXMuIFRoZSBkdXJhdGlvbnMgb2YgbGFrZSBoZWF0d2F2ZXMgaGF2ZSBhbHNvIGluY3JlYXNlZCBhdCBhIHJhdGUgb2YgNy43IGRheXMgZGVjYWRlLTEuIFVuZGVyIHRoZSBoaWdoLWdyZWVuaG91c2UtZ2FzLWVtaXNzaW9uIHNjZW5hcmlvLCBsYWtlIHN1cmZhY2UgdGVtcGVyYXR1cmUgYW5kIGxha2UgaGVhdHdhdmUgZHVyYXRpb24gd2VyZSBwcm9qZWN0ZWQgdG8gaW5jcmVhc2UgYnkgMi4yIMKwQyBhbmQgMTk3IGRheXMgYXQgdGhlIGVuZCBvZiAyMXN0IGNlbnR1cnksIHJlc3BlY3RpdmVseS4gU3VjaCBkcmFzdGljIGNoYW5nZXMgd291bGQgd29yc2VuIHRoZSBlbnZpcm9ubWVudGFsIGNvbmRpdGlvbnMgb2YgbGFrZXMgc3ViamVjdGVkIHRvIGhpZ2ggYW5kIGluY3JlYXNpbmcgYW50aHJvcG9nZW5pYyBwcmVzc3VyZXMsIHBvc2luZyBncmVhdCB0aHJlYXRzIHRvIGFxdWF0aWMgYmlvZGl2ZXJzaXR5IGFuZCBodW1hbiBoZWFsdGguIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjIxMjAzL3JzLjMucnMtMjQ0MjE0Mi92MSIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cucmVzZWFyY2hzcXVhcmUuY29tL2FydGljbGUvcnMtMjQ0MjE0Mi9sYXRlc3QucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjMtMDEtMTciLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTEwIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJSZXNlYXJjaCBTcXVhcmUgKFJlc2VhcmNoIFNxdWFyZSkiLCJpbXBhY3RGYWN0b3IiOiIwLjQwIn0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlJlc2VhcmNoIFNxdWFyZSAoVW5pdGVkIFN0YXRlcykifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjU3LCJjaXRlZEJ5Q291bnQiOjN9fSwicXVvdGUiOiJJZiB0aGUgYm90dG9tIHdhdGVyIHRlbXBlcmF0dXJlIGFsc28gaW5jcmVhc2VzLCBtaW5lcmFsaXphdGlvbiBvZiBvcmdhbmljIG1hdHRlciB3aWxsIGJlIGFjY2VsZXJhdGVkLCB3aGljaCB3aWxsIGxlYWQgdG8gaGlnaGVyIGVtaXNzaW9uIG9mIGNhcmJvbiBkaW94aWRlIGFuZCBtZXRoYW5lNDYsNDcsIHJlaW5mb3JjaW5nIGNsaW1hdGUgd2FybWluZyBkdWUgdG8gZ3JlZW5ob3VzZSBnYXNlcy4gVGhpcyBwcm9jZXNzIHdpbGwgc3BlZWQgdXAgdGhlIHJlY3ljbGluZyBvZiBudXRyaWVudHMgYW5kIGZhdm9yIHBoeXRvcGxhbmt0b24gZ3Jvd3RoLCBzdWdnZXN0aW5nIHBvc2l0aXZlIGZlZWRiYWNrIGJldHdlZW4gY2xpbWF0ZSBjaGFuZ2UgYW5kIGV1dHJvcGhpY2F0aW9uNDguIFRvZ2V0aGVyIHdpdGggdGhlIGhlYXZ5IGV4dGVybmFsIG51dHJpZW50IGxvYWRpbmcgZnJvbSBkb21lc3RpYywgYWdyaWN1bHR1cmFsLCBhbmQgaW5kdXN0cmlhbCB3YXN0ZSwgdGhlIGdyb3d0aCBvZiBwaHl0b3BsYW5rdG9uIGlzIHByb21vdGVkIGJ5IGhvdCB3YXRlciB1bmRlciBudXRyaWVudC1yaWNoIGNvbmRpdGlvbnM0OS4gQmVzaWRlcyB0aGF0LCBzdHJvbmdlciBzdHJhdGnvv71jYXRpb24gd2lsbCBwZXJtaXQgcGh5dG9wbGFua3RvbiB0byBjaXJjdWxhdGUgaW4gdGhlIGV1cGhvdGljIHpvbmU1MC4gSW5jcmVhc2luZyBhbGdhZSBibG9vbXMgaGF2ZSBiZWVuIHJlcG9ydGVkIHdvcmxkd2lkZSwgZXNwZWNpYWxseSBpbiByZWdpb25zIHRoYXQgZGVwZW5kIGhpZ2hseSBvbiBmZXJ0aWxpemVyczUxLiBJbiBDaGluYSwgdGhlIG9jY3VycmVuY2Ugb2YgYWxnYWUgYmxvb21zIGluIHRoZSAyMDEwcyBoYXMgaW5jcmVhc2VkIG5lYXJseSBuaW5lIHRpbWVzIHJlbGF0aXZlIHRvIHRoZSAxOTgwcy0xOTkwczUxLCBhbmQgbWFueSBibG9vbXMgaGF2ZSBiZWVuIHJlcG9ydGVkIHJlY2VudGx5IGluIG5hdHVyYWwgbGFrZXMgYW5kIHJlc2Vydm9pcnMgKFN1cHBsZW1lbnRhcnkgRmlnLiA3KS4gRnVydGhlcm1vcmUsIHRoZSB3YXJtZXIgc3VyZmFjZSB3YXRlciBhbmQgc3Ryb25nZXIgc3RyYXRp77%2B9Y2F0aW9uIHdpbGwgZmF2b3IgdGhlIGRvbWluYW5jZSBvZiBjeWFub2JhY3RlcmlhIGluIGxha2VzIGdpdmVuIGl0cyBoaWdoZXIgb3B0aW1hbCBncm93dGggdGVtcGVyYXR1cmUgYW5kIGFiaWxpdHkgdG8gYWRqdXN0IGJ1b3lhbmN5NDksIGxlYWRpbmcgdG8gaGlnaGVyIGZyZXF1ZW5jeSBvZiBoYXJtZnVsIGFsZ2FlIGJsb29tczQ3LiBUaGUgdG94aWMgc3Vic3RhbmNlcyBwcm9kdWNlZCBieSBjeWFub2JhY3RlcmlhIHdpbGwgdGhyZWF0ZW4gdGhlIHNlY3VyaXR5IG9mIGRyaW5raW5nIHdhdGVyIGFuZCBodW1hbiBoZWFsdGg2LDUyLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjQ1LjEyNjk2MDc1NDM5NDUzLCJsZWZ0Ijo0LjU3NTE2Mjg4NzU3MzI0Miwid2lkdGgiOjkwLjk0MTIwMDI1NjM0NzY2LCJoZWlnaHQiOjEuNzc3MTYzMzg2MzQ0OTA5N319LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjQ3LjQwMTcyMTk1NDM0NTcsImxlZnQiOjQuNTc1MTYyODg3NTczMjQyLCJ3aWR0aCI6OTAuMjA1MjQ1OTcxNjc5NjksImhlaWdodCI6MS43NzcxNjMzODYzNDQ5MDk3fX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NDkuNjQ5MTE2NTE2MTEzMjgsImxlZnQiOjQuNTc1MTYyODg3NTczMjQyLCJ3aWR0aCI6OTEuMDQzNzMxNjg5NDUzMTIsImhlaWdodCI6NC4yNjg4Njk4NzY4NjE1NzJ9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo1NC4zODgyMTc5MjYwMjUzOSwibGVmdCI6NC41NzUxNjI4ODc1NzMyNDIsIndpZHRoIjo0OC40NzYyMjY4MDY2NDA2MjUsImhlaWdodCI6MS45OTQxMDgwODA4NjM5NTI2fX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NTguMzk2NDM0NzgzOTM1NTUsImxlZnQiOjQuNTc1MTYyODg3NTczMjQyLCJ3aWR0aCI6ODcuODgyMzg1MjUzOTA2MjUsImhlaWdodCI6MS43NzcxNjMzODYzNDQ5MDk3fX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NjAuNjQzODI5MzQ1NzAzMTI1LCJsZWZ0Ijo0LjU3NTE2Mjg4NzU3MzI0Miwid2lkdGgiOjkwLjQ1MDU2MTUyMzQzNzUsImhlaWdodCI6MS45OTQxMDgwODA4NjM5NTI2fX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NjMuMTA4MTY5NTU1NjY0MDYsImxlZnQiOjQuNTc1MTYyODg3NTczMjQyLCJ3aWR0aCI6ODkuOTY3NTk3OTYxNDI1NzgsImhlaWdodCI6MS45OTQxMDgwODA4NjM5NTI2fX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NjUuNTcyNDQ4NzMwNDY4NzUsImxlZnQiOjQuNTc1MTYyODg3NTczMjQyLCJ3aWR0aCI6OTAuNTgwODg2ODQwODIwMzEsImhlaWdodCI6MS45OTQxNjk3MTIwNjY2NTA0fX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NjguMDM2Nzg4OTQwNDI5NjksImxlZnQiOjQuNTc1MTYyODg3NTczMjQyLCJ3aWR0aCI6ODkuMTkwNDM3MzE2ODk0NTMsImhlaWdodCI6Ni41NDM2OTMwNjU2NDMzMTA1fX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NzUuMDUwNjUxNTUwMjkyOTcsImxlZnQiOjQuNTc1MTYyODg3NTczMjQyLCJ3aWR0aCI6OTAuMzQ3MDY4Nzg2NjIxMSwiaGVpZ2h0IjoxLjk5NDE2OTcxMjA2NjY1MDR9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo3Ny41MTQ5OTE3NjAyNTM5LCJsZWZ0Ijo0LjU3NTE2Mjg4NzU3MzI0Miwid2lkdGgiOjkwLjIyMjQ4ODQwMzMyMDMxLCJoZWlnaHQiOjEuOTk0MTY5NzEyMDY2NjUwNH19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjc5Ljk3OTMzOTU5OTYwOTM4LCJsZWZ0Ijo0LjU3NTE2Mjg4NzU3MzI0Miwid2lkdGgiOjg1Ljk4MjEzMTk1ODAwNzgxLCJoZWlnaHQiOjEuOTk0MTY5NzEyMDY2NjUwNH19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjgifX1d. Recreational users face acute risks from skin rashes, eye irritation, and respiratory distress upon scum contact, while swimmers ingest aerosols laden with bioavailable toxins (Paerl et al., 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhhbnMgVy4iLCJmYW1pbHkiOiJQYWVybCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLLkUuIiwiZmFtaWx5IjoiSGF2ZW5zIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik4uUy4iLCJmYW1pbHkiOiJIYWxsIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlQuRy4iLCJmYW1pbHkiOiJPdHRlbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNLiIsImZhbWlseSI6IlpodSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJIYWlodWkiLCJmYW1pbHkiOiJYdSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJHLiIsImZhbWlseSI6IlpodSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJCLiIsImZhbWlseSI6IlFpbiJ9fV0sImlkcyI6eyJpc3NuIjpbXSwiaXNibiI6W10sImRvaSI6IjEwLjE3NjE1LzRua2Qtems5MyIsIm9wZW5hbGV4IjoiVzQzMDcwNjI0MjkifSwidGl0bGUiOiJNaXRpZ2F0aW5nIGEgZ2xvYmFsIGV4cGFuc2lvbiBvZiB0b3hpYyBjeWFub2JhY3RlcmlhbCBibG9vbXM6IENvbmZvdW5kaW5nIGVmZmVjdHMgYW5kIGNoYWxsZW5nZXMgcG9zZWQgYnkgY2xpbWF0ZSBjaGFuZ2UiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTc2MTUvNG5rZC16azkzIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2Nkci5saWIudW5jLmVkdS9kb3dubG9hZHMvMDU3NDIyODM0IiwiaXNzdWVkRGF0ZSI6IjIwMjAtMDEtMDEiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTAxIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJDYXJvbGluYSBEaWdpdGFsIFJlcG9zaXRvcnkgKFVuaXZlcnNpdHkgb2YgTm9ydGggQ2Fyb2xpbmEgYXQgQ2hhcGVsIEhpbGwpIn0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlVuaXZlcnNpdHkgb2YgTm9ydGggQ2Fyb2xpbmEgYXQgQ2hhcGVsIEhpbGwifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjAsImNpdGVkQnlDb3VudCI6MH19LCJxdW90ZSI6IlRoYXQgcmVsYXRpb25zaGlwIGhhcyBzZXJ2ZWQgYXMgdGhlIGZvdW5kYXRpb24gZm9yIHNldHRpbmcgIG51dHJpZW50IGxvYWRpbmcgdGFyZ2V0cyBpbiBsYWtlcyBhcm91bmQgdGhlIHdvcmxkLCBhbmQgY2FyZWZ1bGx5ICBkZXNpZ25lZCBleHBlcmltZW50cyBtYXkgZWx1Y2lkYXRlIHRvIHdoYXQgZXh0ZW50IHRlbXBlcmF0dXJlICBpbmNyZWFzZSBhZmZlY3RzIHRoZSB0aHJlc2hvbGQgYXQgd2hpY2ggdGhlIGZyZXF1ZW5jeSBvZiBvY2N1ciAgcmVuY2Ugb2YgQ3lhbm9IQUJzIHF1aWNrbHkgcmlzZXMgd2l0aCBpbmNyZWFzaW5nIE4gYW5kIFAuIENvbmNsdXNpb25zIFJlY2VudCBzdHVkaWVzIGluZGljYXRlIHRoYXQgdGhlIHN5bmVyZ2lzdGljIGNvbWJpbmF0aW9uIG9mIGFudGhyb3BvZ2VuaWMgbnV0cmllbnQgbG9hZGluZywgcmlzaW5nIHRlbXBlcmF0dXJlcywgZW5oYW5jZWQgIHZlcnRpY2FsIHN0cmF0aWZpY2F0aW9uLCBpbmNyZWFzZWQgcmVzaWRlbmNlIHRpbWUgYW5kIG1vcmUgIGV4dHJlbWUgY2xpbWF0aWMgY29uZGl0aW9ucyBvdmVyYWxsIHdpbGwgZmF2b3VyIGN5YW5vYmFjdGVyaWFsICBkb21pbmFuY2UgaW4gYSB3aWRlIHJhbmdlIG9mIGFxdWF0aWMgZWNvc3lzdGVtcy4gVGhlIGV4cGFuICBzaW9uIG9mIEN5YW5vSEFCcyBoYXMgc2VyaW91cyBjb25zZXF1ZW5jZXMgZm9yIGh1bWFuIHdhdGVyICBzdXBwbGllcywgZmlzaGVyaWVzIGFuZCByZWNyZWF0aW9uYWwgcmVzb3VyY2VzLiBFY29zeXN0ZW0gc2NhbGUgIHBoeXNpY2FsIGNoZW1pY2FsIGJpb3RpYyBtaXRpZ2F0aW9uIHN0cmF0ZWdpZXMgd2lsbCBoYXZlIHRvICBpbmNvcnBvcmF0ZSBudXRyaWVudCAobW9zdCBvZnRlbiBib3RoIE4gYW5kIFApIGlucHV0IHJlZHVjdGlvbnMuICBUaGUgbWFnbml0dWRlcyBvZiB0aGVzZSByZWR1Y3Rpb25zIHdpbGwgbmVlZCB0byBiZSBzeXN0ZW0gIHNwZWNpZmljIGFuZCB3aWxsIGxpa2VseSBuZWVkIHRvIGJlIGFkanVzdGVkIHdpdGggY2hhbmdpbmcgY2xpICBtYXRpYyBjb25kaXRpb25zLiBTcGVjaWZpY2FsbHksIHdhdGVyIHF1YWxpdHkgbWFuYWdlcnMgd2lsbCBoYXZlICB0byBhY2NvbW1vZGF0ZSB3YXJtaW5nIGFuZCBncmVhdGVyIGh5ZHJvbG9naWNhbCB2YXJpYWJpbGl0eSAgYW5kIGV4dHJlbWVuZXNzIHJlc3VsdGluZyBmcm9tIGNsaW1hdGUgY2hhbmdlIGluIGZvcm11bGF0aW5nICB0aGVpciBzdHJhdGVnaWVzLiBBIGtleSBsb25nIHRlcm0gY29udHJvbCB3ZSBjYW4gZXhlcnQgdG8gcmVkdWNlICB0aGUgcmF0ZSBhbmQgZXh0ZW50IG9mIGdsb2JhbCB3YXJtaW5nIGlzIGN1cmJpbmcgZ3JlZW5ob3VzZSBnYXMgIGVtaXNzaW9ucy4gV2l0aG91dCB0aGlzIGVzc2VudGlhbCBzdGVwLCBmdXR1cmUgd2FybWluZyB0cmVuZHMsICBoeWRyb2xvZ2ljYWwgZXh0cmVtZXMgKHJlY29yZCByYWluZmFsbHMgYW5kIGRyb3VnaHRzKSBhbmQgdGhlaXIgIGVmZmVjdHMgb24gYXF1YXRpYyBlY29zeXN0ZW1zIHdpbGwgY29udGludWUgdG8gcHJvbW90ZSBvcHBvciAgdHVuaXN0aWMgQ3lhbm9IQUJzLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjoxNC4xNDkyMzM4MTgwNTQyLCJsZWZ0IjoxMy4xMjg1OTUzNTIxNzI4NTIsIndpZHRoIjozNy4wODIxNzIzOTM3OTg4MywiaGVpZ2h0IjoxLjUxNjYwMjI3Nzc1NTczNzN9fSx7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6MTUuNDUxNTA4NTIyMDMzNjkxLCJsZWZ0IjoxMy4xMjg1NDQ4MDc0MzQwODIsIndpZHRoIjozNy4wODQ4MzEyMzc3OTI5NywiaGVpZ2h0IjoxLjUxNjYwMzIzMTQzMDA1Mzd9fSx7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6MTYuNzUzNzg0MTc5Njg3NSwibGVmdCI6MTMuMTI4NTQyOTAwMDg1NDUsIndpZHRoIjozNy4wODMyMDk5OTE0NTUwOCwiaGVpZ2h0IjoxLjUxNjYwNDE4NTEwNDM3MDF9fSx7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6MTguMDU2MDUzMTYxNjIxMDk0LCJsZWZ0IjoxMy4xMjg1NDQ4MDc0MzQwODIsIndpZHRoIjozNi42MTUwNzQxNTc3MTQ4NDQsImhlaWdodCI6MS41MTY2MDQxODUxMDQzNzAxfX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjE5LjQ2ODM3ODA2NzAxNjYsImxlZnQiOjEzLjEyODU0NDgwNzQzNDA4Miwid2lkdGgiOjM0LjI5MDUzMTE1ODQ0NzI2NiwiaGVpZ2h0IjoxLjI0NTAxMTIxMDQ0MTU4OTR9fSx7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6MjIuMjI2MTQyODgzMzAwNzgsImxlZnQiOjEzLjEyODU1MDUyOTQ3OTk4LCJ3aWR0aCI6Ny42NDk4NzM3MzM1MjA1MDgsImhlaWdodCI6MS4xMzU0ODMxNDU3MTM4MDYyfX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjIzLjczNDg0OTkyOTgwOTU3LCJsZWZ0IjoxMy4xMjg1MzUyNzA2OTA5MTgsIndpZHRoIjozNy4wODkyODY4MDQxOTkyMiwiaGVpZ2h0IjoyNi4wODg2NDAyMTMwMTI2OTV9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIxMSJ9fV0%3D. Livestock, reliant on lake water for drinking, suffer mass mortalities; documented cases in Rewalsar Lake link Microcystis blooms to cattle hepatotoxicity and sudden deaths from neurotoxic anatoxins (Kumar & Mahajan, 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlBhd2FuIiwiZmFtaWx5IjoiS3VtYXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW1icmlzaCBLdW1hciIsImZhbWlseSI6Ik1haGFqYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyNTIzLTM5NjMiLCIyNTIzLTM5NzEiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMDcvczQyNDUyLTAyMC0zMDgyLTgiLCJtYWciOiIzMDM1OTYwMjI4Iiwib3BlbmFsZXgiOiJXMzAzNTk2MDIyOCJ9LCJ0aXRsZSI6IlRyb3BoaWMgc3RhdHVzIGFuZCBpdHMgcmVndWxhdGluZyBmYWN0b3IgZGV0ZXJtaW5hdGlvbiBhdCB0aGUgUmV3YWxzYXIgTGFrZSwgbm9ydGh3ZXN0IEhpbWFsYXlhIChIUCksIEluZGlhLCBiYXNlZCBvbiBzZWxlY3RlZCBwYXJhbWV0ZXJzIGFuZCBtdWx0aXZhcmlhdGUgc3RhdGlzdGljYWwgYW5hbHlzaXMiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAwNy9zNDI0NTItMDIwLTMwODItOCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9saW5rLnNwcmluZ2VyLmNvbS9jb250ZW50L3BkZi8xMC4xMDA3L3M0MjQ1Mi0wMjAtMzA4Mi04LnBkZiIsImlzc3VlZERhdGUiOiIyMDIwLTA2LTIyIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiU04gQXBwbGllZCBTY2llbmNlcyIsImlzc3VlIjo3LCJ2b2x1bWUiOjIsImltcGFjdEZhY3RvciI6IjcuMzUifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiU3ByaW5nZXIgTmF0dXJlIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo0MCwiY2l0ZWRCeUNvdW50IjoxOX19LCJxdW90ZSI6IkNoYW5naW5nIHF1YWxpdHkgaGFzIGNyZWF0ZWQgdGhlIHdhdGVyIHNjYXJjaXR5IGFuZCAgYSBodWdlIG1hc3Mgb2YgcG9wdWxhdGlvbiB0aHVzIGJlZW4gZGVwcml2ZWQgZnJvbSB1c2luZyAgdGhlIGZyZXNod2F0ZXIgZm9yIHZhcmlvdXMgcHVycG9zZXMgWzMsIDRdLiBIb3dldmVyLCB0aGUgIG9ibm94aW91cyBtYXRlcmlhbHMgdGhhdCBzdGF5IGluIHdhdGVyIGJvZGllcyBmb3IgYSBsb25nLXRpbWUgcGVyaW9kIGJyaW5nIGNoYW5nZXMgaW4gdGhlIHBoeXNpb2NoZW1pY2FsIGNoYXItYWN0ZXJpc3RpY3MgYXMgd2VsbCBhcyB0aGUgbmF0dXJhbCBzY2VuYXJpbyBvZiB0aGUgbGFrZXMgWzVdLiAgVGhlIEhpbWFsYXlhbiBsYWtlcyBhcmUgdW5kZXIgc3RyZXNzIGR1ZSB0byBtaXNtYW5hZ2VkIGh1bWFuIGFjdGl2aXRpZXMsIHdoaWNoIGluY3JlYXNlIG51dHJpZW50cyBhbmQgcG9sbHV0aW9uIGluIHRlcm1zIG9mIGhlYXZ5IG1ldGFscyB0b3hpY2l0eSBhbmQgYmFjdGVyaWFsIGNvbnRhbWluYXRpb24gIFs24oCTOF0uIFRoZSBtYW5hZ2VtZW50IGFuZCBwcmVzZXJ2YXRpb24gb2YgdGhlIGxha2VzIGF0ICBwcmVzZW50IGhhdmUgYmVjb21lIGEgZ3JlYXQgY2hhbGxlbmdlIGZvciBlbnZpcm9ubWVudGFsLWlzdHMgYW5kIG1hbmFnZXJzIFs1LCA5XS4gVGhlIGV1dHJvcGhpY2F0aW9uIGlzIGFsc28gYSBtYWluIHByb2JsZW0gb2YgdGhlICBmcmVzaHdhdGVyIGxha2VzLCBhbmQgaXQgaXMgc3BlY2lmaWVkIGZvciB0aGUgbGFrZXMgaGF2aW5nICBudXRyaWVudCBlbnJpY2htZW50LiBJdCBjYXVzZXMgZXhjZXNzaXZlIGdyb3d0aCBvZiBhbGdhZSAgYW5kIGlzIGNvbnNpZGVyZWQgdG8gYmUgcmVzcG9uc2libGUgZm9yIGRlZ3JhZGluZyB0aGUgIHF1YWxpdGF0aXZlIG5hdHVyZSBvZiB0aGUgbGFrZXMgWzUsIDEwXS4gVGhlIGV1dHJvcGhpY2F0aW9uICBpcyBmb2xsb3dlZCBieSB0aGUgYWxnYWwgYmxvb20sIHdoaWNoIGdlbmVyYXRlcyB0b3hpbiwgIHdvcnNlbnMgd2F0ZXIgcXVhbGl0eSwgZGVwbGV0ZXMgZGlzc29sdmVkIG94eWdlbiAoRE8pICogUGF3YW4gS3VtYXIsIHBhd2FuLmV2c0BnbWFpbC5jb20gfCAxRGVwYXJ0bWVudCBvZiBFbnZpcm9ubWVudGFsIFNjaWVuY2VzLCBDZW50cmFsIFVuaXZlcnNpdHkgb2YgSGltYWNoYWwgUHJhZGVzaCwgRGhhcmFtc2hhbGEgMTc2MjE1LCBJbmRpYS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2OC41Nzg0NjA2OTMzNTkzOCwibGVmdCI6OC41NzE0MTg3NjIyMDcwMzEsIndpZHRoIjo0MC4zMjc1MTQ2NDg0Mzc1LCJoZWlnaHQiOjEuNTE5ODU1MjYwODQ4OTk5fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NzAuMTU4OTk2NTgyMDMxMjUsImxlZnQiOjguNTY5NzM4Mzg4MDYxNTIzLCJ3aWR0aCI6NDAuMzU0NzI0ODg0MDMzMiwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjcxLjczOTU0NzcyOTQ5MjE5LCJsZWZ0Ijo4LjU2OTczODM4ODA2MTUyMywid2lkdGgiOjQwLjM0OTY5MzI5ODMzOTg0NCwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjczLjMyMDA5MTI0NzU1ODYsImxlZnQiOjguNTY4MDU5OTIxMjY0NjQ4LCJ3aWR0aCI6NDAuMDIwMDg4MTk1ODAwNzgsImhlaWdodCI6MS41MTk4NTUyNjA4NDg5OTl9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo3NC45MDA2NDIzOTUwMTk1MywibGVmdCI6OC41NjgwNTk5MjEyNjQ2NDgsIndpZHRoIjozOS45OTk5Mjc1MjA3NTE5NSwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjc2LjQ4MTE4NTkxMzA4NTk0LCJsZWZ0Ijo4LjU2ODA1NzA2MDI0MTcsIndpZHRoIjo0MC4zNTcwNzQ3Mzc1NDg4MywiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjc4LjA2MTcyOTQzMTE1MjM0LCJsZWZ0Ijo4LjU2OTczNjQ4MDcxMjg5LCJ3aWR0aCI6NDAuMzUzNzEzOTg5MjU3ODEsImhlaWdodCI6MS41MTk4NTUyNjA4NDg5OTl9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2MC42NzU3Mjc4NDQyMzgyOCwibGVmdCI6NTEuNDI3MTY1OTg1MTA3NDIsIndpZHRoIjo0MC4zNzg1NjY3NDE5NDMzNiwiaGVpZ2h0IjoxOC45MDU4NTUxNzg4MzMwMDh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo4MS45MjczMzc2NDY0ODQzOCwibGVmdCI6OC40MDE0OTc4NDA4ODEzNDgsIndpZHRoIjo3My45NTQ2OTY2NTUyNzM0NCwiaGVpZ2h0IjoyLjY3ODc2MjY3NDMzMTY2NX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEifX1d. Bioaccumulation in aquatic food webs transfers toxins to humans via fish consumption, with Dolichospermum-derived saxitoxins mimicking paralytic shellfish poisoning (Jilbert et al., 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRvbSIsImZhbWlseSI6IkppbGJlcnQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUmFvdWzigJBNYXJpZSIsImZhbWlseSI6IkNvdXR1cmUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQnJpYW4iLCJmYW1pbHkiOiJIdXNlciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLYWxldmkiLCJmYW1pbHkiOiJTYWxvbmVuIn19XSwiaWRzIjp7Imlzc24iOlsiMDAxOC04MTU4IiwiMTU3My01MTE3Il0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDA3L3MxMDc1MC0wMjAtMDQ0NTcteCIsIm1hZyI6IjMxMDA0NTkwNTAiLCJvcGVuYWxleCI6IlczMTAwNDU5MDUwIn0sInRpdGxlIjoiUHJlZmFjZTogUmVzdG9yYXRpb24gb2YgZXV0cm9waGljIGxha2VzOiBjdXJyZW50IHByYWN0aWNlcyBhbmQgZnV0dXJlIGNoYWxsZW5nZXMiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAwNy9zMTA3NTAtMDIwLTA0NDU3LXgiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vbGluay5zcHJpbmdlci5jb20vY29udGVudC9wZGYvMTAuMTAwNy9zMTA3NTAtMDIwLTA0NDU3LXgucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjAtMTEtMTYiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA3IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJIeWRyb2Jpb2xvZ2lhIiwiaXNzdWUiOjIxLCJ2b2x1bWUiOjg0NywiaW1wYWN0RmFjdG9yIjoiMy4wMCJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjQzNDMsImxhc3RQYWdlIjo0MzU3fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlNwcmluZ2VyIFNjaWVuY2UrQnVzaW5lc3MgTWVkaWEifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjk1LCJjaXRlZEJ5Q291bnQiOjU4fX0sInF1b3RlIjoiU2ltaWxhciBkZWdyYWRhdGlvbiBvZiBmcmVzaHdhdGVyIGVjb3N5c3RlbXMgaXMgb2JzZXJ2ZWQgd29ybGR3aWRlLCB3aXRoIHRoZSB0aW1lbGluZSBvZiBodW1hbiBwcmVzc3VyZSBkaWN0YXRlZCBieSB0cmVuZHMgaW4gdXJiYW5pemF0aW9uIGFuZCBpbnRlbnNpdmUgYWdyaWN1bHR1cmUgaW4gZGlmZmVyZW50IGdlb2dyYXBoaWMgcmVnaW9ucyAoZS5nLiwgVmVubmVtbyBldCBhbC4sIDIwMDkpLiBUaGUgcHJpbmNpcGFsIGRyaXZlciBvZiBlY29zeXN0ZW0gc3RyZXNzIGluIG1vc3QgYXF1YXRpYyBzeXN0ZW1zIGlzIGV1dHJvcGhpY2F0aW9uLCB0aGUgZXhjZXNzIGdyb3d0aCBvZiBhbGdhZSBkcml2ZW4gYnkgYW50aHJvcG9nZW5pYyBpbnB1dHMgb2YgdGhlIG1hY3JvbnV0cmllbnRzIG5pdHJvZ2VuIChOKSBhbmQgcGhvc3Bob3J1cyAoUCkgKFNtaXRoLCAyMDAzOyBCaXJrIGV0IGFsLiwgMjAyMCkuIE51dHJpZW50cyBhcmUgZW1pdHRlZCBmcm9tIGJvdGggcG9pbnQgYW5kIGRpZmZ1c2Ugc291cmNlcyBpbiB0aGUgaHVtYW4gZW52aXJvbm1lbnQgYW5kIGVudGVyIGFxdWF0aWMgc3lzdGVtcyB2aWEgc3VyZmFjZSBydW5vZmYsIGFzIHdlbGwgYXMgZ3JvdW5kLXdhdGVyIGFuZCBhdG1vc3BoZXJpYyBkZXBvc2l0aW9uIChQYWVybCwgMTk5NykuIEV1dHJvcGhpY2F0aW9uIG9mdGVuIGxlYWRzIHRvIHRoZSBkb21pbmFuY2Ugb2YgcHJpbWFyeSBwcm9kdWN0aW9uIGJ5IGN5YW5vYmFjdGVyaWEsIGluY2x1ZGluZyB0b3hpYyBnZW5lcmEgc3VjaCBhcyBEb2xpY2hvc3Blcm11bSwgQXBoYW5pem9tZW5vbiwgYW5kIE1pY3JvY3lzdGlzIChIdWlzbWFuIGV0IGFsLiwgMjAxOCksIGFzIHdlbGwgYXMgb3h5Z2VuIGRlcGxldGlvbiBhbmQgaGFiaXRhdCBkZWdyYWRhdGlvbiBpbiBsYWtlcyAoQXJlbmQgZXQgYWwuLCAyMDExKS4gQmVjYXVzZSBsYWtlcyBwcm92aWRlIGEgcmFuZ2Ugb2YgY3J1Y2lhbCBlY29zeXN0ZW0gc2VydmljZXMgKFdpbHNvbiAmIENhcnBlbnRlciwgMTk5OSksIHRoZXJlIGlzIGEgZ3Jvd2luZyBuZWVkIGZvciBtYW5hZ2VtZW50IGFwcHJvYWNoZXMgdGhhdCByZXN0b3JlIHdhdGVyIHF1YWxpdHkgaW4gZnJlc2h3YXRlciBzeXN0ZW1zLiBXaXRoaW4gdGhlIGZyYW1ld29yayBvZiBVbml0ZWQgTmF0aW9ucyBTdXN0YWluYWJsZSBEZXZlbG9wLW1lbnQgR29hbCA2IChDbGVhbiBXYXRlciBhbmQgU2FuaXRhdGlvbiksIHNldmVyYWwgZGlzdGluY3QgdGFyZ2V0cyByZWxhdGUgdG8gdGhlIGltcHJvdmVtZW50IG9mIHdhdGVyIHF1YWxpdHkgYW5kIHByb3RlY3Rpb24gb2YgYXF1YXRpYyBlY29zeXN0ZW1zIGJ5IDIwMzAgKFVuaXRlZCBOYXRpb25zLCAyMDE1KS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyOC4yOTg4NjQzNjQ2MjQwMjMsImxlZnQiOjguNTQ5MjA1NzgwMDI5Mjk3LCJ3aWR0aCI6MzkuMzcxNDk0MjkzMjEyODksImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MjkuOTkxMTM0NjQzNTU0Njg4LCJsZWZ0Ijo4LjU0OTIwNTc4MDAyOTI5Nywid2lkdGgiOjM5LjM2NTExOTkzNDA4MjAzLCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMxLjY4MzQwNjgyOTgzMzk4NCwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNjk2NjMyMzg1MjUzOSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozMy4zNjc5NzMzMjc2MzY3MiwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNzU2NzkwMTYxMTMyOCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNS4wNjAyNDkzMjg2MTMyOCwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNzA5NjQwNTAyOTI5NywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNi43NTI1MTc3MDAxOTUzMSwibGVmdCI6OC41NDkyMDg2NDEwNTIyNDYsIndpZHRoIjozOS4zNzY5NjA3NTQzOTQ1MywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC40MzcwODgwMTI2OTUzMSwibGVmdCI6OC41NDkyMDg2NDEwNTIyNDYsIndpZHRoIjozOS4zNjY1Njk1MTkwNDI5NywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0MC4xMjkzNTYzODQyNzczNDQsImxlZnQiOjguNTQ5MjA4NjQxMDUyMjQ2LCJ3aWR0aCI6MzkuMzY0Mzc5ODgyODEyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0MS44MjE2MjQ3NTU4NTkzNzUsImxlZnQiOjguNTQ5MjA4NjQxMDUyMjQ2LCJ3aWR0aCI6MzkuMzY1NjczMDY1MTg1NTUsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDMuNTA2MTk1MDY4MzU5Mzc1LCJsZWZ0Ijo4LjU0OTM4OTgzOTE3MjM2Mywid2lkdGgiOjM5LjM3MDM5MTg0NTcwMzEyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0NS4xOTg0NjM0Mzk5NDE0MDYsImxlZnQiOjguNTQ5NTcyOTQ0NjQxMTEzLCJ3aWR0aCI6MzkuMzY1ODQ0NzI2NTYyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ni44OTA3MzU2MjYyMjA3LCJsZWZ0Ijo4LjU0OTU3Mjk0NDY0MTExMywid2lkdGgiOjM5LjM3Mjk1NTMyMjI2NTYyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0OC41NzUzMDIxMjQwMjM0NCwibGVmdCI6OC41NDk1NzI5NDQ2NDExMTMsIndpZHRoIjozOS4zNzI0MjEyNjQ2NDg0NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1MC4yNjc1NzgxMjUsImxlZnQiOjguNTQ5NTc2NzU5MzM4Mzc5LCJ3aWR0aCI6MzkuMzcxMTI0MjY3NTc4MTI1LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjUxLjk1OTg0MjY4MTg4NDc2NiwibGVmdCI6OC41NDk1NzY3NTkzMzgzNzksIndpZHRoIjozOS4zNjc4MzU5OTg1MzUxNTYsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTMuNTc0MTE5NTY3ODcxMDk0LCJsZWZ0Ijo4LjU0OTU3Njc1OTMzODM3OSwid2lkdGgiOjM5LjM3MDc3NzEzMDEyNjk1LCJoZWlnaHQiOjEuMzM2ODk1NTg1MDYwMTE5Nn19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjU1LjI2NjM5MTc1NDE1MDM5LCJsZWZ0Ijo4LjU0OTc2MDgxODQ4MTQ0NSwid2lkdGgiOjM5LjM2NjU4MDk2MzEzNDc2NiwiaGVpZ2h0IjoxLjMzNjg5NTU4NTA2MDExOTZ9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1Ny4wMjg5NTM1NTIyNDYwOTQsImxlZnQiOjguNTQ5OTQ0ODc3NjI0NTEyLCJ3aWR0aCI6MzkuMzYyMDI2MjE0NTk5NjEsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTguNzEzNTIzODY0NzQ2MDk0LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjkuMjk5OTgyMDcwOTIyODUyLCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjYwLjQwNTc5MjIzNjMyODEyNSwibGVmdCI6MTAuNjIyNDU1NTk2OTIzODI4LCJ3aWR0aCI6MzcuMjg3ODY4NDk5NzU1ODYsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NjIuMDk4MDYwNjA3OTEwMTU2LCJsZWZ0Ijo4LjU0OTk0MzkyMzk1MDE5NSwid2lkdGgiOjM5LjM3MzMyNTM0NzkwMDM5LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjYzLjc4MjYzMDkyMDQxMDE1NiwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzMxMjY5ODM2NDI1OCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2NS40NzQ5MDY5MjEzODY3MiwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNjUxMTk5MzQwODIwMywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2Ny4xNjcxNzUyOTI5Njg3NSwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNjk0ODAxMzMwNTY2NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2OC43ODE0NDgzNjQyNTc4MSwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzczMzg0MDk0MjM4MywiaGVpZ2h0IjoxLjMzNjg5MTUzMTk0NDI3NX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjcwLjU0NDAxMzk3NzA1MDc4LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjM5LjM2OTMwODQ3MTY3OTY5LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjcyLjIzNjI4OTk3ODAyNzM0LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjM5LjM3MjAzMjE2NTUyNzM0NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo3My45MjA4NDUwMzE3MzgyOCwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzEzMDczNzMwNDY4NzUsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNDM0NCJ9fV0%3D.

Acute human health effects manifest as gastrointestinal distress, fever, and headaches at low doses, escalating to severe hepatotoxicity, neurotoxic paralysis, and cardiotoxicity at higher exposures. Chronic low-level ingestion promotes tumor promotion and genotoxicity, classified as probable carcinogens by IARC (Meerhoff et al., 2022)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik0uIiwiZmFtaWx5IjoiTWVlcmhvZmYifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSi4iLCJmYW1pbHkiOiJBdWRldCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJULkEuIiwiZmFtaWx5IjoiRGF2aWRzb24ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTC4gRGUiLCJmYW1pbHkiOiJNZWVzdGVyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlMuIiwiZmFtaWx5IjoiSGlsdCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTLiIsImZhbWlseSI6Iktvc3RlbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJaYWlodWEiLCJmYW1pbHkiOiJMaXUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTi4iLCJmYW1pbHkiOiJNYXp6ZW8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSGFucyBXLiIsImZhbWlseSI6IlBhZXJsIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik0uIiwiZmFtaWx5IjoiU2NoZWZmZXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiRS4iLCJmYW1pbHkiOiJKZXBwZXNlbiJ9fV0sImlkcyI6eyJpc3NuIjpbXSwiaXNibiI6W10sImRvaSI6IjEwLjE3NjE1Lzd2ZTUtdzAxNiIsIm9wZW5hbGV4IjoiVzQzMTg0MTU1NzcifSwidGl0bGUiOiJGZWVkYmFjayBiZXR3ZWVuIGNsaW1hdGUgY2hhbmdlIGFuZCBldXRyb3BoaWNhdGlvbjogcmV2aXNpdGluZyB0aGUgYWxsaWVkIGF0dGFjayBjb25jZXB0IGFuZCBob3cgdG8gc3RyaWtlIGJhY2siLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTc2MTUvN3ZlNS13MDE2IiwicmVzb3VyY2VVcmwiOiJodHRwczovL2Nkci5saWIudW5jLmVkdS9kb3dubG9hZHMvaDk4OXJjOTM4IiwiaXNzdWVkRGF0ZSI6IjIwMjItMDEtMDEiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTEwIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJDYXJvbGluYSBEaWdpdGFsIFJlcG9zaXRvcnkgKFVuaXZlcnNpdHkgb2YgTm9ydGggQ2Fyb2xpbmEgYXQgQ2hhcGVsIEhpbGwpIiwiaW1wYWN0RmFjdG9yIjoiMC4xMCJ9LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJVbml2ZXJzaXR5IG9mIE5vcnRoIENhcm9saW5hIGF0IENoYXBlbCBIaWxsIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjowLCJjaXRlZEJ5Q291bnQiOjN9fSwicXVvdGUiOiJXaXRoIGNsaW1hdGUgd2FybWluZywgdGhlIGRpcmVjdCBhbmQgaW5kaS1yZWN0IGVmZmVjdHMgb2YgaW5jcmVhc2VkIGxha2UgdGVtcGVyYXR1cmVzIHdpbGwgbGlrZWx5IGRyaXZlIGNoYW5nZXMgaW4gY3lhbm9iYWN0ZXJpYWwgdGF4YSBhbmQgdGhlaXIgcHJvZHVjLXRpb24gb2YgY3lhbm9iYWN0ZXJpYWwgdG94aW5zLCBsaWtlbHkgaW5jbHVkaW5nIHByb21vdC1pbmcgdGhlIHNlbGVjdGlvbiBvZiBhIGZldyBNaWNyb2N5c3RpcyBzcGVjaWVzIG9yIHN0cmFpbnMgcHJvZHVjaW5nIGhpZ2hseSB0b3hpYyBtaWNyb2N5c3Rpbi1MUiAoTWFudHpvdWtpIGV0IGFsLiAyMDE4KS4gRGVzcGl0ZSB2YXJpYXRpb25zIGFtb25nIGxha2VzIGluIGEgd2F0ZXItc2NhcGUsIGEgcmVnaW9uYWwgaW5jcmVhc2UgaW4gbWljcm9jeXN0aW4gbWF4aW1hIGFuZCBkdXJhdGlvbiBvZiBibG9vbXMgaGFzIGFsc28gYmVlbiBjbGVhcmx5IGV4cGxhaW5lZCBieSBpbmNyZWFzaW5nIHRlbXBlcmF0dXJlIGluIGNlbnRyYWwgTm9ydGggQW1lcmljYSAoSGF5ZXMgZXQgYWwuIDIwMjApLCB3aXRoIGluY3JlYXNpbmcgcmlza3MgdG8gaHVtYW4gaGVhbHRoLiBHcm93aW5nIGV2aWRlbmNlIGluZGljYXRlcyB0aGF0IGN5YW5vYmFjdGVyaWFsIGRvbWluYW5jZSB3aWxsIGV4ZXJ0IGEgcG9zaXRpdmUgZmVlZGJhY2sgd2l0aCBjbGltYXRlIHdhcm1pbmcgKEhhdmVucyBldCBhbC4gMjAxNiwgUGFlcmwgZXQgYWwuIDIwMjApLiBNYW55IGN5YW5vYmFjdGVyaWFsIHNwZWNpZXMgcHJvbGlmZXJhdGUgb24gdGhlIGxha2Ugc3VyZmFjZSwgYWJzb3JiaW5nIGxpZ2h0LCBlbmhhbmNpbmcgbG9jYWxpemVkIHRlbXBlcmF0dXJlIGluY3JlYXNlcywgYW5kIGluY3JlYXNpbmcgdGhlcm1hbCBzdHJhdGlmaWNhdGlvbiBwb3RlbnRpYWwgKEthaHJ1IGV0IGFsLiAxOTkzKS4gQ3lhbm9iYWN0ZXJpYSBwcm9tb3RlIHRoZSByZWxlYXNlIG9mIEdIR3MsIHBhcnRpY3VsYXJseSBDSDQgKFlhbiBldCBhbC4gMjAxNyksIGR1cmluZyB0aGUgZGVjb21wb3NpdGlvbiBmb2xsb3dpbmcgdGhlIGNvbGxhcHNlIG9mIGEgYmxvb20uIEN5YW5vYmFjdGVyaWEgYWxzbyBwcm9tb3RlIHJlbWluZXJhbGl6YXRpb24gb2YgbnV0cmktZW50cyBkdXJpbmcgYmxvb21zLCBsZWFkaW5nIHRvIG11bHRpcGxlIGJsb29tcyBpbiBhIHNpbmdsZSBzZWFzb24uIFVuZGVyIGV1dHJvcGhpYyBhbmQgd2FybWluZyBjb25kaS10aW9ucywgY3lhbm9iYWN0ZXJpYSBkb21pbmFuY2UgbWF5IHRodXMgYmUgYSBzZWxmLXBlcnBldHVhdGluZyBwcm9waGVjeSAoRmlnLiAyIGFuZCAzKS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo2Ny4wNjM0MDAyNjg1NTQ2OSwibGVmdCI6NTEuNDc4ODAxNzI3Mjk0OTIsIndpZHRoIjozOC42NzM0MjM3NjcwODk4NDQsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6NjguNjk4NTQ3MzYzMjgxMjUsImxlZnQiOjUxLjQ3ODgwMTcyNzI5NDkyLCJ3aWR0aCI6MzguNjc5NDk2NzY1MTM2NzIsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6NzAuMzMzNjg2ODI4NjEzMjgsImxlZnQiOjUxLjQ3ODgwMTcyNzI5NDkyLCJ3aWR0aCI6MzguNjc3NTgxNzg3MTA5Mzc1LCJoZWlnaHQiOjEuNDcxMTAxNTIyNDQ1Njc4N319LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjcxLjk2ODgyNjI5Mzk0NTMxLCJsZWZ0Ijo1MS40Nzg4MDE3MjcyOTQ5Miwid2lkdGgiOjM4LjY3OTEwMzg1MTMxODM2LCJoZWlnaHQiOjEuNDcxMTAxNTIyNDQ1Njc4N319LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjczLjYxMDU3MjgxNDk0MTQsImxlZnQiOjUxLjQ3ODgwMTcyNzI5NDkyLCJ3aWR0aCI6MzguNjc0NDI3MDMyNDcwNywiaGVpZ2h0IjoxLjQ3Mzc0Njg5NTc5MDF9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo3NS4yNDcwMzk3OTQ5MjE4OCwibGVmdCI6NTEuNDc4ODA5MzU2Njg5NDUsIndpZHRoIjozOC42NzQyNjMwMDA0ODgyOCwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo3Ni44ODIxODY4ODk2NDg0NCwibGVmdCI6NTEuNDc4ODA5MzU2Njg5NDUsIndpZHRoIjozOC42ODExNzE0MTcyMzYzMywiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo3OC41MjUyNTMyOTU4OTg0NCwibGVmdCI6NTEuNDc4ODA5MzU2Njg5NDUsIndpZHRoIjozOC42NzU2NjI5OTQzODQ3NjYsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODAuMTYwNDAwMzkwNjI1LCJsZWZ0Ijo1MS40Nzg4MDkzNTY2ODk0NSwid2lkdGgiOjM4LjY4MjAyOTcyNDEyMTA5NCwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo4MS43OTU1Mzk4NTU5NTcwMywibGVmdCI6NTEuNDc4ODA5MzU2Njg5NDUsIndpZHRoIjozOC42NzQ5ODM5NzgyNzE0ODQsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODMuNDM4NjEzODkxNjAxNTYsImxlZnQiOjUxLjQ3ODgwOTM1NjY4OTQ1LCJ3aWR0aCI6MzguNjc2MTg1NjA3OTEwMTU2LCJoZWlnaHQiOjEuNDg2OTczNTI0MDkzNjI4fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODUuMDg5NjMwMTI2OTUzMTIsImxlZnQiOjUxLjQ3ODgwOTM1NjY4OTQ1LCJ3aWR0aCI6NC42ODI4MDUwNjEzNDAzMzIsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODYuNzI0Nzc3MjIxNjc5NjksImxlZnQiOjUzLjQ0MTE3NzM2ODE2NDA2LCJ3aWR0aCI6Ni4xNDAzNTU1ODcwMDU2MTUsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODYuNzI0ODkxNjYyNTk3NjYsImxlZnQiOjYwLjk5NDg3Njg2MTU3MjI2Niwid2lkdGgiOjYuMDM2OTkyNTQ5ODk2MjQsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODYuNzI0ODkxNjYyNTk3NjYsImxlZnQiOjY4LjQ0NjUyNTU3MzczMDQ3LCJ3aWR0aCI6Ni4xNTA3MDU4MTQzNjE1NzIsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODYuNzI0ODkxNjYyNTk3NjYsImxlZnQiOjc2LjAwOTgyNjY2MDE1NjI1LCJ3aWR0aCI6Mi43MjczMzMzMDcyNjYyMzU0LCJoZWlnaHQiOjEuNDcxMTAxNTIyNDQ1Njc4N319LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjg2LjcyNDg5MTY2MjU5NzY2LCJsZWZ0Ijo4MC4xNDk1ODE5MDkxNzk2OSwid2lkdGgiOjEwLjAwODIzNDk3NzcyMjE2OCwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo4OC4zNjAwMzg3NTczMjQyMiwibGVmdCI6NTEuNDc3NjU3MzE4MTE1MjM0LCJ3aWR0aCI6MzguNjcyMDU0MjkwNzcxNDg0LCJoZWlnaHQiOjEuNDcxMTAxNTIyNDQ1Njc4N319LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjkwLjAwMzExMjc5Mjk2ODc1LCJsZWZ0Ijo1MS40Nzc2NTczMTgxMTUyMzQsIndpZHRoIjozOC42NzY0OTg0MTMwODU5NCwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo5MS42MzgyNTIyNTgzMDA3OCwibGVmdCI6NTEuNDc3NjUzNTAzNDE3OTcsIndpZHRoIjozOC42NzU4MzQ2NTU3NjE3MiwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjksImJib3giOnsidG9wIjo2LjQ2ODkzNTk2NjQ5MTY5OSwibGVmdCI6OS44NDAxMzM2NjY5OTIxODgsIndpZHRoIjozOC42ODYzMjUwNzMyNDIxOSwiaGVpZ2h0IjoxNi4yMTIxMjU3NzgxOTgyNDJ9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI5In19XQ%3D%3D. In Chilika Lagoon, post-bloom toxin surges have correlated with elevated hospital admissions for liver ailments and dermatitis among fishers (Nair & Nayak, 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hdnlhIFZpa3JhbWFuIiwiZmFtaWx5IjoiTmFpciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQcmF0ZWVwIEt1bWFyIiwiZmFtaWx5IjoiTmF5YWsifX1dLCJpZHMiOnsiaXNzbiI6WyIyMjk2LTc3NDUiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzODkvZm1hcnMuMjAyMy4xMDg3Mjk2Iiwib3BlbmFsZXgiOiJXNDM4MjcyMzEzMyJ9LCJ0aXRsZSI6IlVuY292ZXJpbmcgd2F0ZXIgcXVhbGl0eSBhbmQgZXZhbHVhdGluZyB2dWxuZXJhYmlsaXRpZXMgb2Ygc21hbGwtc2NhbGUgZmlzaGVyaWVzIGluIENoaWxpa2EgTGFnb29uLCBJbmRpYSIsImFic3RyYWN0IjoiU21hbGwtc2NhbGUgZmlzaGVyaWVzIChTU0ZzKSBwbGF5IGEgc3Ryb25nIHJvbGUgaW4gc3VzdGFpbmluZyBtaWxsaW9ucyBvZiBsaXZlbGlob29kcywgZm9vZCBzZWN1cml0eSwgbnV0cml0aW9uLCBhbmQgaW5jb21lIGdsb2JhbGx5IGJ1dCB0aGUgZmlzaGVycyBlbmdhZ2VkIGluIHRoaXMgc2VjdG9yIHNpbXVsdGFuZW91c2x5IGV4cGVyaWVuY2UgaGlnaCBsZXZlbHMgb2YgdnVsbmVyYWJpbGl0eSBhbmQgcHJvY2Vzc2VzIG9mIG1hcmdpbmFsaXNhdGlvbi4gU2V2ZXJhbCBmYWN0b3JzIGFyZSBhdHRyaWJ1dGVkIHRvIHRoZSBtdWx0aWRpbWVuc2lvbmFsIHZ1bG5lcmFiaWxpdGllcyB0aGUgc21hbGwtc2NhbGUgZmlzaGVycyBleHBlcmllbmNlLCBzcGFubmluZyBib3RoIG5hdHVyYWwgKGUuZy4sIG5hdHVyYWwgZGlzYXN0ZXJzLCBlY29zeXN0ZW0gY2hhbmdlKSBhbmQgYW50aHJvcG9nZW5pYyAoZS5nLiwgcG9saWN5IGNoYW5nZSwgaHlkcm9sb2dpY2FsIGludGVydmVudGlvbnMsIGFxdWFjdWx0dXJlKSBwcmVzc3VyZXMuIFdoaWxlIHRoZXJlIGlzIG11Y2ggbGl0ZXJhdHVyZSBvbiB2YXJpb3VzIG5hdHVyYWwgYW5kIGh1bWFuIGRyaXZlcnMgb2YgdnVsbmVyYWJpbGl0eSBpbiBzbWFsbC1zY2FsZSBmaXNoZXJ5IGNvbW11bml0aWVzLCBhbiBhYnNlbmNlIG9mIHJlc2VhcmNoIGNvbm5lY3RpbmcgdnVsbmVyYWJpbGl0eSB3aXRoIHdhdGVyIHF1YWxpdHkgaXMgZXZpZGVudC4gRmlzaGVyIGNvbW11bml0aWVzIG9mdGVuIHRhbGsgYWJvdXQgZmlzaCBpbiByZWxhdGlvbiB0byB0aGUgaGVhbHRoIG9mIHRoZWlyIGFxdWF0aWMgaGFiaXRhdHMgd2hlcmVpbiB3YXRlciBxdWFsaXR5IGlzIHNlZW4gYXMgYSBrZXkgcGFyYW1ldGVyLiBUaGUgbGluayBiZXR3ZWVuIGhlYWx0aHkgZmlzaCBhbmQgZ29vZCBxdWFsaXR5IHdhdGVyIGhhcyBzaWduaWZpY2FudCBpbXBsaWNhdGlvbnMgZm9yIHN0cm9uZyBhbmQgdmlhYmxlIGZpc2hpbmcgY29tbXVuaXRpZXMuIFRoaXMgcGFwZXIgZXhhbWluZXMgdGhlc2UgbGlua3MgZnVydGhlciBieSBmb2N1c2luZyBvbiB0aGUgbmF0dXJlIG9mIHZ1bG5lcmFiaWxpdGllcyBjYXVzZWQgYnkgd2F0ZXIgcXVhbGl0eSBjaGFuZ2VzIGluIHRoZSBzbWFsbC1zY2FsZSBmaXNoZXJ5IHN5c3RlbSBvZiBDaGlsaWthIExhZ29vbiBpbiBJbmRpYS4gV2UgdW5kZXJ0YWtlIGRldGFpbGVkIGFuYWx5c2lzIG9mIHRoZSBpbnZhc2l2ZSBzaHJpbXAgYXF1YWN1bHR1cmUgYWN0aXZpdGllcyBhbmQgaHlkcm9sb2dpY2FsIGludGVydmVudGlvbnMgZm9yIG9wZW5pbmcgb2YgYSBsYWdvb24gaW5sZXQgd2l0aCB0aGUgQmF5IG9mIEJlbmdhbCBhcyB0d28gZG9taW5hbnQgZHJpdmVycyBhZHZlcnNlbHkgaW1wYWN0aW5nIHdhdGVyIHF1YWxpdHkgYW5kIGluY3JlYXNpbmcgdnVsbmVyYWJpbGl0aWVzIG9mIHRoZSBlbnRpcmUgc21hbGwtc2NhbGUgZmlzaGVyaWVzIHNvY2lhbC1lY29sb2dpY2FsIHN5c3RlbS4gT3VyIGFuYWx5c2lzIHN1Z2dlc3RzIHRoYXQgdGhlcmUgYXJlIHN0cm9uZyBpbnRlcmNvbm5lY3Rpb25zIGJldHdlZW4gY2hhbmdlcyBpbiB3YXRlciBxdWFsaXR5IGFuZCB0aGUgbGV2ZWxzIG9mIHZ1bG5lcmFiaWxpdGllcyBpbiB0aGUgU1NGcyBvZiBDaGlsaWthIExhZ29vbi4gUG9sbHV0YW50cyBzdWNoIGFzIHBlc3RpY2lkZXMsIGFuZCBvcmdhbmljIGNvbXBvdW5kcyBhY2N1bXVsYXRlIGluIGZpc2ggdGlzc3VlcyBhbmQgYWZmZWN0IHRoZWlyIGdyb3d0aCwgcmVwcm9kdWN0aW9uLCBhbmQgb3ZlcmFsbCBoZWFsdGguIFRoaXMgbGVkIHRvIGRlY2xpbmVzIGluIGZpc2ggcG9wdWxhdGlvbnMsIG1ha2luZyBpdCBtb3JlIGRpZmZpY3VsdCBmb3IgZmlzaGVycyB0byBtYWtlIGEgbGl2aW5nLiBJbiBhZGRpdGlvbiB0byBkaXJlY3QgaW1wYWN0cyBvbiBmaXNoIHBvcHVsYXRpb25zLCBwb29yIHdhdGVyIHF1YWxpdHkgYWxzbyBoYXMgaW5kaXJlY3QgZWZmZWN0cyBvbiB0aGUgc29jaWFsIGFuZCBlY29ub21pYyB2dWxuZXJhYmlsaXR5IG9mIFNTRnMuIEZvciBleGFtcGxlLCBjb250YW1pbmF0aW9uIG9mIHdhdGVyIHNvdXJjZXMgbGVkIHRvIHRoZSByZWR1Y2VkIG51bWJlciBvZiBmaXNoIHNwZWNpZXMgcmVkdWNpbmcgdGhlIGFtb3VudCBvZiB0aW1lIGZpc2hlcnMgY2FuIHNwZW5kIG9uIGZpc2hpbmcgYWN0aXZpdGllcy4gVGhpcyBhbHNvIGFmZmVjdGVkIHRoZSBtYXJrZXRhYmlsaXR5IG9mIGZpc2ggcHJvZHVjdHMsIHJlZHVjZWQgaW5jb21lIGFuZCBpbmNyZWFzZWQgcG92ZXJ0eS4gVG8gZnVsbHkgdW5kZXJzdGFuZCB0aGUgaW50ZXJjb25uZWN0aW9ucyBiZXR3ZWVuIHdhdGVyIHF1YWxpdHkgYW5kIHZ1bG5lcmFiaWxpdHkgaW4gU1NGcyBpbiBDaGlsaWthIExhZ29vbiwgaXQgaXMgaW1wb3J0YW50IHRvIGNvbnNpZGVyIGJvdGggZW52aXJvbm1lbnRhbCBhbmQgc29jaWFsIGZhY3RvcnMsIGFzIHdlbGwgYXMgdGhlIGNvbXBsZXggZmVlZGJhY2sgbG9vcHMgYmV0d2VlbiB0aGVzZSBmYWN0b3JzLiBUaGUgc3R1ZHkgaGVscHMgaW4gYnJpZGdpbmcgYSBjcnVjaWFsIGdhcCBpbiBvdXIgdW5kZXJzdGFuZGluZyBvZiB0aGUgcm9sZSBvZiB3YXRlciBxdWFsaXR5IGluIHZ1bG5lcmFiaWxpdHkgYW5hbHlzaXMgd2l0aGluIHJlc291cmNlIGRlcGVuZGVudCBjb21tdW5pdGllcy4gV2UgY29uY2x1ZGUgd2l0aCBrZXkgaW5zaWdodHMgb24gcG9zc2libGUgY29waW5nIHJlc3BvbnNlcyBhbmQgYWRhcHRpdmUgY2FwYWNpdHkgbmVjZXNzYXJ5IGZvciB0aGUgc21hbGwtc2NhbGUgZmlzaGVyaWVzIGNvbW11bml0aWVzIHRvIHRyYW5zaXRpb24gdG93YXJkIHZpYWJpbGl0eS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMzM4OS9mbWFycy4yMDIzLjEwODcyOTYiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vd3d3LmZyb250aWVyc2luLm9yZy9hcnRpY2xlcy8xMC4zMzg5L2ZtYXJzLjIwMjMuMTA4NzI5Ni9wZGYiLCJpc3N1ZWREYXRlIjoiMjAyMy0wNi0yOSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZyb250aWVycyBpbiBNYXJpbmUgU2NpZW5jZSIsInZvbHVtZSI6MTAsImltcGFjdEZhY3RvciI6IjQuMjgifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRnJvbnRpZXJzIE1lZGlhIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo1MSwiY2l0ZWRCeUNvdW50Ijo1fX0sInF1b3RlIjoiV2F0ZXIgcXVhbGl0eSBvZiBDaGlsaWthIExhZ29vbiBoYXMgZGVncmFkZWQgZHVlIHRvIGFudGhyb3BvZ2VuaWMgaW50ZXJ2ZW50aW9ucyBhbmQgbmF0dXJhbCBmb3JjZXMgb2YgY2hhbmdlLiBWYXJpb3VzIGFjdGl2aXRpZXMgaW4gdGhlIENoaWxpa2EgTGFnb29uLCBzdWNoIGFzIGluZHVzdHJpYWwgd2FzdGV3YXRlciBkaXNwb3NhbCwgc2V3YWdlIGR1bXBpbmcsIGFxdWFjdWx0dXJlLCBoeWRyb2xvZ2ljYWwgaW50ZXJ2ZW50aW9ucywgYW5kIGN5Y2xvbmVzLCBhcmUgcHJvZHVjaW5nIHNhbGluaXR5IGNoYW5nZXMsIHNlZGltZW50IGRlcG9zaXRpb24sIG51dHJpZW50IGVucmljaG1lbnQsIGV1dHJvcGhpY2F0aW9uLCBhbmQgZGVhZCB6b25lcywgYXMgc3RhdGVkLiBGaXNoIHN1cnZpdmFsIGlzIGF0IHJpc2sgYXMgYSByZXN1bHQgb2YgdGhlIHNpZ25pZmljYW50IHJlZHVjdGlvbiBpbiBlbnZpcm9ubWVudGFsIGNpcmN1bXN0YW5jZXMsIHdoaWNoIGRpbWluaXNoZXMgZmlzaGVybWVu4oCZcyBpbmNvbWUgYW5kIHB1c2hlcyB0aGVtIG91dCBvZiBzb2NpZXR5LiBUaGVzZSBjYXVzZXMgYWZmZWN0aW5nIHRoZSB3YXRlciBxdWFsaXR5IG9mIHRoZSBMYWdvb24gZGVwaWN0IG1hbnkgZmFjZXMgb2YgdnVsbmVyYWJpbGl0eSBpbiBTU0YgY29tbXVuaXRpZXMuIEFzIHRoZSBDaGlsaWthIExhZ29vbiBlbnZpcm9ubWVudCBpcyBleHBlY3RlZCB0byB1bmRlcmdvIGluY3JlYXNpbmdseSBkcmFzdGljIGNoYW5nZXMgKHN1Y2ggYXMgY2F0Y2ggY29tcG9zaXRpb24sIGNhdGNoIGNhcGFjaXR5LCBiaW9kaXZlcnNpdHkgdmFyaWF0aW9ucywgd2F0ZXIgcXVhbGl0eSwgYW5kIGZpc2hlcnkgcmV2ZW51ZXMpLCBpbnRlcm5hdGlvbmFsIHBvbGljeSBoYXMgYmVjb21lIG1vcmUgaW50ZXJlc3RlZCBpbiBhZGFwdGF0aW9uIG1lYXN1cmVzLiBHaXZlbiBob3cgZGl2ZXJzZSBTU0YgZWNvc3lzdGVtcyBhcmUgZXhwZWN0ZWQgdG8gYmUgYmV0d2VlbiBjb250aW5lbnRzLCB0aGVyZSBpcyBubyDigJxvbmUgc2l6ZSBmaXRzIGFsbOKAnSBhcHByb2FjaC4gVGhlIHBhcGVyIG91dGxpbmVkIHNldmVyYWwgY29waW5nIGFuZCBhZGFwdGF0aW9uIG1lY2hhbmlzbXMgdG8gaW1wcm92ZSB3YXRlciBxdWFsaXR5IGFuZCBjb250aW51ZSBmaXNoaW5nIGluIGEgc3VzdGFpbmFibGUgd2F5LiBBZGFwdGF0aW9uIHN0cmF0ZWdpZXMgY2FuIG9jY2FzaW9uYWxseSBoYXZlIHVuZm9yZXNlZW4gY29uc2VxdWVuY2VzIGZvciBmaXNoZXJpZXMgYW5kIGZpc2hpbmcgY29tbXVuaXRpZXMuIEFueSB1bmV4cGVjdGVkIGNvbnNlcXVlbmNlcyBvZiBhZGFwdGF0aW9uIGNhbiBiZSBtaXRpZ2F0ZWQgdGhyb3VnaCBsb25nLXRlcm0gcGxhbm5pbmcgYW5kIGlkZW50aWZpY2F0aW9uIG9mIGZ1dHVyZSBvcHRpb25zIHRocm91Z2ggc2NlbmFyaW8gYW5hbHlzaXMuIFRoZSBmaW5kaW5ncyBvZiB0aGlzIHBhcGVyIGNhbiBiZSB1dGlsaXplZCB0byBkZXZlbG9wIHBvbGljaWVzIHRvIHByb3RlY3Qgd2F0ZXIgcXVhbGl0eSBhbmQgU1NGcyBpbiBhIHZhcmlldHkgb2YgY29hc3RhbCBsYWdvb24gc2V0dGluZ3MuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjIyLjI4NTAzNjA4NzAzNjEzMywibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ5MzA1MzQzNjI3OTMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjIzLjc3MzUxOTUxNTk5MTIxLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDgzMzY0MTA1MjI0NjEsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjI1LjI1NTU4NDcxNjc5Njg3NSwibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ4OTU1OTE3MzU4Mzk4NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjYuNzQ0MDY4MTQ1NzUxOTUzLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNTA2ODA1NDE5OTIxODc1LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyOC4yMjYxMzMzNDY1NTc2MTcsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40OTA0NjMyNTY4MzU5NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjkuNzE0NjE2Nzc1NTEyNjk1LCJsZWZ0Ijo4LjMzMTU5MjU1OTgxNDQ1Mywid2lkdGgiOjQwLjQ5NDA5ODY2MzMzMDA4LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozMS4xOTY2ODM4ODM2NjY5OTIsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40OTQzOTIzOTUwMTk1MywiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MzIuNjg1MTY1NDA1MjczNDQsImxlZnQiOjguMzMxNTk2Mzc0NTExNzE5LCJ3aWR0aCI6NDAuNDg3NDQ1ODMxMjk4ODMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM0LjE2NzIzMjUxMzQyNzczNCwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40OTc3NDU1MTM5MTYwMTYsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM1LjY1NTcxNTk0MjM4MjgxLCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ5ODYzODE1MzA3NjE3LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozNy4xMzc3ODMwNTA1MzcxMSwibGVmdCI6MTAuNjg1MTg2Mzg2MTA4Mzk4LCJ3aWR0aCI6MzguMTQxMTA1NjUxODU1NDcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM4LjYyNjI2NjQ3OTQ5MjE5LCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4NzE1MjA5OTYwOTM3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDAuMTA4MzMzNTg3NjQ2NDg0LCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4OTg1NjcxOTk3MDcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQxLjU5NjgxMzIwMTkwNDMsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDg4NjY2NTM0NDIzODMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQzLjA3ODg4MDMxMDA1ODU5NCwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40ODE4NjExMTQ1MDE5NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDQuNTY3MzYzNzM5MDEzNjcsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDgzMDU4OTI5NDQzMzYsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQ2LjA0OTQzMDg0NzE2Nzk3LCJsZWZ0Ijo4LjMzMzEwODkwMTk3NzUzOSwid2lkdGgiOjQwLjUwMDc0NzY4MDY2NDA2LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo0Ny41Mzc5MTQyNzYxMjMwNSwibGVmdCI6OC4zMzMxMDg5MDE5Nzc1MzksIndpZHRoIjo0MC40OTU2MDU0Njg3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDkuMDE5OTgxMzg0Mjc3MzQ0LCJsZWZ0Ijo4LjMzMzEwNjA0MDk1NDU5LCJ3aWR0aCI6NDAuNTA2ODE2ODY0MDEzNjcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUwLjUwODQ2NDgxMzIzMjQyLCJsZWZ0Ijo4LjMzMzEwNjA0MDk1NDU5LCJ3aWR0aCI6NDAuNDg4NTA2MzE3MTM4NjcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUxLjk5MDUyODEwNjY4OTQ1LCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDk3NzI2NDQwNDI5NjksImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUzLjQ3OTAxMTUzNTY0NDUzLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6MzMuMTU5NDQ2NzE2MzA4NTk0LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo1NC45NjEwNzg2NDM3OTg4MywibGVmdCI6MTAuNjgzNjcwMDQzOTQ1MzEyLCJ3aWR0aCI6MzguMTM2NTk2Njc5Njg3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NTYuNDQ5NTYyMDcyNzUzOTA2LCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDg3MTQ0NDcwMjE0ODQ0LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEzIn19XQ%3D%3D. Livestock losses compound vulnerabilities for rural communities, disrupting milk and meat supplies. Vulnerable populations children, elderly, and immunocompromised face heightened risks, with Indian studies reporting microcystin-LR in 70% of eutrophic lake samples, underscoring systemic threats to public health infrastructure (Gobler et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkNocmlzdG9waGVyIEouIiwiZmFtaWx5IjoiR29ibGVyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlJ1dGggVy4iLCJmYW1pbHkiOiJEcmlua3dhdGVyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkEuIFcuIiwiZmFtaWx5IjoiQW50aG9ueSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJKZW5uaWZlciBBLiIsImZhbWlseSI6IkdvbGVza2kifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW5uIE1hcmllIEUuIiwiZmFtaWx5IjoiRmFtdWxhcm8tUGVjb3JhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1pY2hhZWwgQi4iLCJmYW1pbHkiOiJXYWxsYWNlIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5vcmEgUi4gVy4iLCJmYW1pbHkiOiJTdHJhcXVhZGluZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJSb25vam95IiwiZmFtaWx5IjoiSGVtIn19XSwiaWRzIjp7Imlzc24iOlsiMTY2NC0zMDJYIl0sImlzYm4iOltdLCJkb2kiOiIxMC4zMzg5L2ZtaWNiLjIwMjQuMTQ2NDY4NiIsInBtaWQiOiIzOTU2ODk5NyIsIm9wZW5hbGV4IjoiVzQ0MDQxMjA5NTIiLCJleGEiOiJodHRwczovL3d3dy5mcm9udGllcnNpbi5vcmcvam91cm5hbHMvbWljcm9iaW9sb2d5L2FydGljbGVzLzEwLjMzODkvZm1pY2IuMjAyNC4xNDY0Njg2L2Z1bGwifSwidGl0bGUiOiJTZXdhZ2UtYW5kIGZlcnRpbGl6ZXItZGVyaXZlZCBudXRyaWVudHMgYWx0ZXIgdGhlIGludGVuc2l0eSwgZGl2ZXJzaXR5LCBhbmQgdG94aWNpdHkgb2YgaGFybWZ1bCBjeWFub2JhY3RlcmlhbCBibG9vbXMgaW4gZXV0cm9waGljIGxha2VzIiwiYWJzdHJhY3QiOiJDeWFub2JhY3RlcmlhbCBoYXJtZnVsIGFsZ2FsIGJsb29tcyAoQ0hBQnMpIGFyZSBwcm9tb3RlZCBieSBleGNlc3NpdmUgbnV0cmllbnQgbG9hZGluZyBhbmQsIHdoaWxlIGZlcnRpbGl6ZXJzIGFuZCBzZXdhZ2UgYXJlIHRoZSBtb3N0IHByZXZhbGVudCBleHRlcm5hbCBudXRyaWVudCBzb3VyY2VzIGluIG1vc3Qgd2F0ZXJzaGVkcywgdGhlIGRpZmZlcmVudGlhbCBlZmZlY3RzIG9mIHRoZXNlIG51dHJpZW50IHNvdXJjZXMgb24gQ0hBQnMgYXJlIHVua25vd24uIEhlcmUsIHdlIHRyYWNrZWQgQ0hBQnMgYW5kIHBlcmZvcm1lZCBleHBlcmltZW50cyBpbiBmaXZlIGRpc3RpbmN0IGxha2VzIGFjcm9zcyB0aGUgTm9ydGhlcm4gVVMgaW5jbHVkaW5nIExha2UgRXJpZS4gRmVydGlsaXplcnMgd2l0aCBhbW1vbml1bSBhbmQgb3J0aG9waG9zcGhhdGUsIG1lbWJyYW5lICgwLjIgzrxtKS1maWx0ZXJlZCBzZXdhZ2UgKGRvbWluYXRlZCBieSByZWR1Y2VkIGZvcm1zIG9mIG5pdHJvZ2VuKSBzYW5kLWFuZCBtZW1icmFuZS1maWx0ZXJlZCBzZXdhZ2UgKGRvbWluYXRlZCBieSBuaXRyYXRlKSwgYW5kIGFuIGlub3JnYW5pYyBudXRyaWVudCBzb2x1dGlvbiBvZiBhbW1vbml1bSBhbmQgb3J0aG9waG9zcGhhdGUgd2VyZSB1c2VkIGFzIGV4cGVyaW1lbnRhbCBudXRyaWVudCBzb3VyY2VzIGZvciBDSEFCcyBhdCBOLWVxdWl2YWxlbnQsIGVudmlyb25tZW50YWxseSByZWFsaXN0aWMgY29uY2VudHJhdGlvbnMuIFBoeXRvcGxhbmt0b24gY29tbXVuaXRpZXMgd2VyZSBldmFsdWF0ZWQgZmx1b3JvbWV0cmljYWxseSwgbWljcm9zY29waWNhbGx5LCBhbmQgdmlhIGhpZ2ggdGhyb3VnaHB1dCBzZXF1ZW5jaW5nIG9mIHRoZSAxNlMgclJOQSBnZW5lLCBhbmQgbGV2ZWxzIG9mIG1pY3JvY3lzdGluIGFuZCB0aGUgzrQgMTUgTiBjb250ZW50IG9mIHBhcnRpY3VsYXRlIG9yZ2FuaWMgbml0cm9nZW4gKM60UE8gMTUgTikgd2VyZSBxdWFudGlmaWVkLiBGZXJ0aWxpemVyIGFuZCBib3RoIHNvdXJjZXMgb2Ygd2FzdGV3YXRlciBpbmNyZWFzZWQgdGhlIGFidW5kYW5jZSBvZiBjeWFub2JhY3RlcmlhIGluIGFsbCBleHBlcmltZW50cyBhY3Jvc3MgYWxsIGZpdmUgbGFrZXMgKCBwICZhbXA7bHQ7IDAuMDUgZm9yIGFsbCkgd2hlcmVhcyBlZmZlY3RzIG9uIGV1a2FyeW90aWMgcGh5dG9wbGFua3RvbiB3ZXJlIGxpbWl0ZWQuIFNhbmQtZmlsdGVyZWQgc2V3YWdlIGNvbnRhaW5lZCBsZXNzIFAsIG9yZ2FuaWMgbWF0dGVyLCBhbmQgYW1tb25pdW0gYnV0IG1vcmUgbml0cmF0ZSBhbmQgaGFkIGEgMjUlIGxlc3MgcG90ZW50IHN0aW11bGF0b3J5IGVmZmVjdCBvbiBjeWFub2JhY3RlcmlhIHRoYW4gbWVtYnJhbmUtZmlsdGVyZWQgc2V3YWdlLCBzdWdnZXN0aW5nIG5pdHJpZmljYXRpb24gbWF5IHBsYXkgYSByb2xlIGluIHJlZHVjaW5nIENIQUJzLiBGZXJ0aWxpemVyIGluY3JlYXNlZCBtaWNyb2N5c3RpbiBsZXZlbHMgYW5kIGRlY3JlYXNlZCB0aGUgzrRQTyAxNSBOIHdoZXJlYXMgd2FzdGV3YXRlciBpbmNyZWFzZWQgzrRQTyAxNSBOICggcCAmYW1wO2x0OyAwLjA1IGZvciBhbGwpLiBNaWNyb2N5c3RpcyB3YXMgdGhlIGdlbnVzIG1vc3QgY29uc2lzdGVudGx5IHByb21vdGVkIGJ5IG51dHJpZW50IHNvdXJjZXMgKCBwICZhbXA7bHQ7IDAuMDUgaW4gYWxsIGV4cGVyaW1lbnRzKSwgZm9sbG93ZWQgYnkgQ3lhbm9iaXVtICggcCAmYW1wO2x0OyAwLjA1IGluIDUwJSBvZiBleHBlcmltZW50cyksIHdpdGggaW5jcmVhc2VzIGluIE1pY3JvY3lzdGlzIGJpb21hc3MgY29uc2lzdGVudGx5IGVsaWNpdGVkIGJ5IG1lbWJyYW5lLWZpbHRlcmVkIHdhc3Rld2F0ZXIuIENvbGxlY3RpdmVseSwgcmVzdWx0cyBkZW1vbnN0cmF0ZSB0aGF0IGRpZmZlcmluZyB0eXBlcyBvZiBzZXdhZ2UgZGlzY2hhcmdlIGFuZCBmZXJ0aWxpemVycyBjYW4gcHJvbW90ZSBDSEFCIGludGVuc2l0eSBhbmQgdG94aWNpdHksIHdoaWxlIGNvbmN1cnJlbnRseSBhbHRlcmluZyBDSEFCIGRpdmVyc2l0eSBhbmQgzrRQTyAxNSBOLiBXaGlsZSBtZW1icmFuZS1maWx0ZXJlZCBzZXdhZ2UgY29uc2lzdGVudGx5IGZhdm9yZWQgTWljcm9jeXN0aXMgLCB0aGUgZGlzY2hhcmdlIG9mIHNld2FnZSB0aHJvdWdoIHNhbmRzIG11dGVkIGJsb29tIGludGVuc2l0eSBzdWdnZXN0aW5nIHNhbmQtYmVkcyBtYXkgcmVwcmVzZW50IGEgdG9vbCB0byByZW1vdmUga2V5IG51dHJpZW50cyBhbmQgcGFydGlhbGx5IG1pdGlnYXRlIENIQUJzLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzg5L2ZtaWNiLjIwMjQuMTQ2NDY4NiIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cuZnJvbnRpZXJzaW4ub3JnL2pvdXJuYWxzL21pY3JvYmlvbG9neS9hcnRpY2xlcy8xMC4zMzg5L2ZtaWNiLjIwMjQuMTQ2NDY4Ni9wZGYiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMS0wNiIsImFjY2Vzc2VkRGF0ZSI6IjIwMjYtMDIiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZyb250aWVycyBpbiBNaWNyb2Jpb2xvZ3kiLCJ2b2x1bWUiOjE1LCJpbXBhY3RGYWN0b3IiOiIzLjk5In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTQ2NDY4NiwibGFzdFBhZ2UiOjE0NjQ2ODZ9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRnJvbnRpZXJzIE1lZGlhIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo3MywiY2l0ZWRCeUNvdW50Ijo5fX0sInF1b3RlIjoiZGVyaXZlZCBudXRyaWVudHMgYWx0ZXIgdGhlIGludGVuc2l0eSwgZGl2ZXJzaXR5LCBhbmQgdG94aWNpdHkgb2YgaGFybWZ1bCBjeWFub2JhY3RlcmlhbCBibG9vbXMgLi4uIEN5YW5vYmFjdGVyaWFsIGhhcm1mdWwgYWxnYWwgYmxvb21zIChDSEFCcykgYXJlIHByb21vdGVkIGJ5IGV4Y2Vzc2l2ZSBudXRyaWVudCBsb2FkaW5nIGFuZCwgd2hpbGUgZmVydGlsaXplcnMgYW5kIHNld2FnZSBhcmUgdGhlIG1vc3QgcHJldmFsZW50IGV4dGVybmFsIG51dHJpZW50IHNvdXJjZXMgaW4gbW9zdCB3YXRlcnNoZWRzLCB0aGUgZGlmZmVyZW50aWFsIGVmZmVjdHMgb2YgdGhlc2UgbnV0cmllbnQgLi4uIG9yZ2FuaWMgbml0cm9nZW4gKM60UE8xNU4pIHdlcmUgcXVhbnRpZmllZC4gRmVydGlsaXplciBhbmQgYm90aCBzb3VyY2VzIG9mIHdhc3Rld2F0ZXIgaW5jcmVhc2VkIHRoZSBhYnVuZGFuY2Ugb2YgLi4uIHBoeXRvcGxhbmt0b24gd2VyZSBsaW1pdGVkLiBTYW5kLWZpbHRlcmVkIHNld2FnZSBjb250YWluZWQgbGVzcyBQLCBvcmdhbmljIG1hdHRlciwgYW5kIGFtbW9uaXVtIGJ1dCBtb3JlIG5pdHJhdGUgYW5kIC4uLiBkaWZmZXJpbmcgdHlwZXMgb2Ygc2V3YWdlIGRpc2NoYXJnZSBhbmQgZmVydGlsaXplcnMgY2FuIHByb21vdGUgQ0hBQiBpbnRlbnNpdHkgYW5kIHRveGljaXR5LCB3aGlsZSBjb25jdXJyZW50bHkgLi4uIHNld2FnZSB0aHJvdWdoIHNhbmRzIG11dGVkIGJsb29tIGludGVuc2l0eSBzdWdnZXN0aW5nIHNhbmQtYmVkcyBtYXkgcmVwcmVzZW50IGEgdG9vbCB0byByZW1vdmUga2V5IG51dHJpZW50cyBhbmQgLi4uICggV2F0c29uIGV0IGFsLiwgMjAxNiA7IEJ1bGxlcmphaG4gZXQgYWwuLCAyMDE2IDsgSHVpc21hbiBldCBhbC4sIDIwMTggKSBhbmQgZXhjZXNzaXZlIG51dHJpZW50IGxvYWRpbmcgaXMgdGhlIHByb2Nlc3MgbW9zdCBjb21tb25seSBjaXRlZCBhcyB0aGUgZW52aXJvbm1lbnRhbCBmYWN0b3IgcHJvbW90aW5nIENIQUJzICgifV0%3D.

Mitigating these risks demands integrated strategies beyond bloom suppression. Advanced water treatment like activated carbon and ozonation degrades cyanotoxins, yet affordability limits adoption in India (Paerl et al., 2019)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhhbnMgVy4iLCJmYW1pbHkiOiJQYWVybCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLYXJsIEUuIiwiZmFtaWx5IjoiSGF2ZW5zIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hdGhhbiBTLiIsImZhbWlseSI6IkhhbGwifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiVGltb3RoeSBHLiIsImZhbWlseSI6Ik90dGVuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1lbmd5dWFuIiwiZmFtaWx5IjoiWmh1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhhaSIsImZhbWlseSI6Ilh1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikd1YW5nd2VpIiwiZmFtaWx5IjoiWmh1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkJvcWlhbmciLCJmYW1pbHkiOiJRaW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIxMzIzLTE2NTAiLCIxNDQ4LTYwNTkiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwNzEvbWYxODM5MiIsIm1hZyI6IjI5NDIzMjE1NDIiLCJvcGVuYWxleCI6IlcyOTQyMzIxNTQyIn0sInRpdGxlIjoiTWl0aWdhdGluZyBhIGdsb2JhbCBleHBhbnNpb24gb2YgdG94aWMgY3lhbm9iYWN0ZXJpYWwgYmxvb21zOiBjb25mb3VuZGluZyBlZmZlY3RzIGFuZCBjaGFsbGVuZ2VzIHBvc2VkIGJ5IGNsaW1hdGUgY2hhbmdlIiwiYWJzdHJhY3QiOiJNYW5hZ2luZyBhbmQgbWl0aWdhdGluZyB0aGUgZ2xvYmFsIGV4cGFuc2lvbiBvZiB0b3hpYyBjeWFub2JhY3RlcmlhbCBoYXJtZnVsIGFsZ2FsIGJsb29tcyAoQ3lhbm9IQUJzKSBpcyBhIG1ham9yIGNoYWxsZW5nZSBmYWNpbmcgcmVzZWFyY2hlcnMgYW5kIHdhdGVyIHJlc291cmNlIG1hbmFnZXJzLiBWYXJpb3VzIGFwcHJvYWNoZXMsIGluY2x1ZGluZyBudXRyaWVudCBsb2FkIHJlZHVjdGlvbiwgYXJ0aWZpY2lhbCBtaXhpbmcgYW5kIGZsdXNoaW5nLCBvbW5pdm9yb3VzIGZpc2ggcmVtb3ZhbCwgYWxnYWVjaWRlIGFwcGxpY2F0aW9ucyBhbmQgc2VkaW1lbnQgZHJlZGdpbmcsIGhhdmUgYmVlbiB1c2VkIHRvIHJlZHVjZSBibG9vbSBvY2N1cnJlbmNlcy4gSG93ZXZlciwgbWFuYWdlcnMgbm93IGZhY2UgdGhlIGFkZGl0aW9uYWwgY2hhbGxlbmdlIG9mIGhhdmluZyB0byBhZGRyZXNzIHRoZSBlZmZlY3RzIG9mIGNsaW1hdGUgY2hhbmdlIG9uIHdhdGVyc2hlZCBoeWRyb2xvZ2ljYWwgYW5kIG51dHJpZW50IGxvYWQgZHluYW1pY3MsIHdhdGVyIHRlbXBlcmF0dXJlLCBtaXhpbmcgcmVnaW1lIGFuZCBpbnRlcm5hbCBudXRyaWVudCBjeWNsaW5nLiBSaXNpbmcgdGVtcGVyYXR1cmVzIGFuZCBpbmNyZWFzaW5nIGZyZXF1ZW5jaWVzIGFuZCBtYWduaXR1ZGVzIG9mIGV4dHJlbWUgd2VhdGhlciBldmVudHMsIGluY2x1ZGluZyB0cm9waWNhbCBjeWNsb25lcywgZXh0cmF0cm9waWNhbCBzdG9ybXMsIGZsb29kcyBhbmQgZHJvdWdodHMsIGFsbCBwcm9tb3RlIEN5YW5vSEFCcyBhbmQgYWZmZWN0IHRoZSBlZmZpY2FjeSBvZiBlY29zeXN0ZW0gcmVtZWRpYXRpb24gbWVhc3VyZXMuIFRoZXNlIGNsaW1hdGljIGNoYW5nZXMgd2lsbCBsaWtlbHkgcmVxdWlyZSBzZXR0aW5nIHN0cmljdGVyIG51dHJpZW50IChpbmNsdWRpbmcgYm90aCBuaXRyb2dlbiBhbmQgcGhvc3Bob3J1cykgcmVkdWN0aW9uIHRhcmdldHMgZm9yIGJsb29tIGNvbnRyb2wgaW4gYWZmZWN0ZWQgd2F0ZXJzLiBJbiBhZGRpdGlvbiwgdGhlIGVmZmljYWN5IG9mIGN1cnJlbnRseSB1c2VkIG1ldGhvZHMgdG8gcmVkdWNlIEN5YW5vSEFCcyB3aWxsIG5lZWQgdG8gYmUgcmUtZXZhbHVhdGVkIGluIGxpZ2h0IG9mIHRoZSBzeW5lcmdpc3RpYyBlZmZlY3RzIG9mIGNsaW1hdGUgY2hhbmdlIHdpdGggbnV0cmllbnQgZW5yaWNobWVudC4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTA3MS9tZjE4MzkyIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2Nkci5saWIudW5jLmVkdS9kb3dubG9hZHMvMDU3NDIyODM0IiwiaXNzdWVkRGF0ZSI6IjIwMTktMDMtMjUiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTEwIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJNYXJpbmUgYW5kIEZyZXNod2F0ZXIgUmVzZWFyY2giLCJpc3N1ZSI6NSwidm9sdW1lIjo3MSwiaW1wYWN0RmFjdG9yIjoiMi43MCJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjU3OSwibGFzdFBhZ2UiOjU3OX19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJDU0lSTyBQdWJsaXNoaW5nIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjoxMDQsImNpdGVkQnlDb3VudCI6OTF9fSwicXVvdGUiOiJUaGF0IHJlbGF0aW9uc2hpcCBoYXMgc2VydmVkIGFzIHRoZSBmb3VuZGF0aW9uIGZvciBzZXR0aW5nICBudXRyaWVudCBsb2FkaW5nIHRhcmdldHMgaW4gbGFrZXMgYXJvdW5kIHRoZSB3b3JsZCwgYW5kIGNhcmVmdWxseSAgZGVzaWduZWQgZXhwZXJpbWVudHMgbWF5IGVsdWNpZGF0ZSB0byB3aGF0IGV4dGVudCB0ZW1wZXJhdHVyZSAgaW5jcmVhc2UgYWZmZWN0cyB0aGUgdGhyZXNob2xkIGF0IHdoaWNoIHRoZSBmcmVxdWVuY3kgb2Ygb2NjdXIgIHJlbmNlIG9mIEN5YW5vSEFCcyBxdWlja2x5IHJpc2VzIHdpdGggaW5jcmVhc2luZyBOIGFuZCBQLiBDb25jbHVzaW9ucyBSZWNlbnQgc3R1ZGllcyBpbmRpY2F0ZSB0aGF0IHRoZSBzeW5lcmdpc3RpYyBjb21iaW5hdGlvbiBvZiBhbnRocm9wb2dlbmljIG51dHJpZW50IGxvYWRpbmcsIHJpc2luZyB0ZW1wZXJhdHVyZXMsIGVuaGFuY2VkICB2ZXJ0aWNhbCBzdHJhdGlmaWNhdGlvbiwgaW5jcmVhc2VkIHJlc2lkZW5jZSB0aW1lIGFuZCBtb3JlICBleHRyZW1lIGNsaW1hdGljIGNvbmRpdGlvbnMgb3ZlcmFsbCB3aWxsIGZhdm91ciBjeWFub2JhY3RlcmlhbCAgZG9taW5hbmNlIGluIGEgd2lkZSByYW5nZSBvZiBhcXVhdGljIGVjb3N5c3RlbXMuIFRoZSBleHBhbiAgc2lvbiBvZiBDeWFub0hBQnMgaGFzIHNlcmlvdXMgY29uc2VxdWVuY2VzIGZvciBodW1hbiB3YXRlciAgc3VwcGxpZXMsIGZpc2hlcmllcyBhbmQgcmVjcmVhdGlvbmFsIHJlc291cmNlcy4gRWNvc3lzdGVtIHNjYWxlICBwaHlzaWNhbCBjaGVtaWNhbCBiaW90aWMgbWl0aWdhdGlvbiBzdHJhdGVnaWVzIHdpbGwgaGF2ZSB0byAgaW5jb3Jwb3JhdGUgbnV0cmllbnQgKG1vc3Qgb2Z0ZW4gYm90aCBOIGFuZCBQKSBpbnB1dCByZWR1Y3Rpb25zLiAgVGhlIG1hZ25pdHVkZXMgb2YgdGhlc2UgcmVkdWN0aW9ucyB3aWxsIG5lZWQgdG8gYmUgc3lzdGVtICBzcGVjaWZpYyBhbmQgd2lsbCBsaWtlbHkgbmVlZCB0byBiZSBhZGp1c3RlZCB3aXRoIGNoYW5naW5nIGNsaSAgbWF0aWMgY29uZGl0aW9ucy4gU3BlY2lmaWNhbGx5LCB3YXRlciBxdWFsaXR5IG1hbmFnZXJzIHdpbGwgaGF2ZSAgdG8gYWNjb21tb2RhdGUgd2FybWluZyBhbmQgZ3JlYXRlciBoeWRyb2xvZ2ljYWwgdmFyaWFiaWxpdHkgIGFuZCBleHRyZW1lbmVzcyByZXN1bHRpbmcgZnJvbSBjbGltYXRlIGNoYW5nZSBpbiBmb3JtdWxhdGluZyAgdGhlaXIgc3RyYXRlZ2llcy4gQSBrZXkgbG9uZyB0ZXJtIGNvbnRyb2wgd2UgY2FuIGV4ZXJ0IHRvIHJlZHVjZSAgdGhlIHJhdGUgYW5kIGV4dGVudCBvZiBnbG9iYWwgd2FybWluZyBpcyBjdXJiaW5nIGdyZWVuaG91c2UgZ2FzICBlbWlzc2lvbnMuIFdpdGhvdXQgdGhpcyBlc3NlbnRpYWwgc3RlcCwgZnV0dXJlIHdhcm1pbmcgdHJlbmRzLCAgaHlkcm9sb2dpY2FsIGV4dHJlbWVzIChyZWNvcmQgcmFpbmZhbGxzIGFuZCBkcm91Z2h0cykgYW5kIHRoZWlyICBlZmZlY3RzIG9uIGFxdWF0aWMgZWNvc3lzdGVtcyB3aWxsIGNvbnRpbnVlIHRvIHByb21vdGUgb3Bwb3IgIHR1bmlzdGljIEN5YW5vSEFCcy4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6MTQuMTQ5MjMzODE4MDU0MiwibGVmdCI6MTMuMTI4NTk1MzUyMTcyODUyLCJ3aWR0aCI6MzcuMDgyMTcyMzkzNzk4ODMsImhlaWdodCI6MS41MTY2MDIyNzc3NTU3MzczfX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjE1LjQ1MTUwODUyMjAzMzY5MSwibGVmdCI6MTMuMTI4NTQ0ODA3NDM0MDgyLCJ3aWR0aCI6MzcuMDg0ODMxMjM3NzkyOTcsImhlaWdodCI6MS41MTY2MDMyMzE0MzAwNTM3fX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjE2Ljc1Mzc4NDE3OTY4NzUsImxlZnQiOjEzLjEyODU0MjkwMDA4NTQ1LCJ3aWR0aCI6MzcuMDgzMjA5OTkxNDU1MDgsImhlaWdodCI6MS41MTY2MDQxODUxMDQzNzAxfX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjE4LjA1NjA1MzE2MTYyMTA5NCwibGVmdCI6MTMuMTI4NTQ0ODA3NDM0MDgyLCJ3aWR0aCI6MzYuNjE1MDc0MTU3NzE0ODQ0LCJoZWlnaHQiOjEuNTE2NjA0MTg1MTA0MzcwMX19LHsicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjoxOS40NjgzNzgwNjcwMTY2LCJsZWZ0IjoxMy4xMjg1NDQ4MDc0MzQwODIsIndpZHRoIjozNC4yOTA1MzExNTg0NDcyNjYsImhlaWdodCI6MS4yNDUwMTEyMTA0NDE1ODk0fX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjIyLjIyNjE0Mjg4MzMwMDc4LCJsZWZ0IjoxMy4xMjg1NTA1Mjk0Nzk5OCwid2lkdGgiOjcuNjQ5ODczNzMzNTIwNTA4LCJoZWlnaHQiOjEuMTM1NDgzMTQ1NzEzODA2Mn19LHsicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjoyMy43MzQ4NDk5Mjk4MDk1NywibGVmdCI6MTMuMTI4NTM1MjcwNjkwOTE4LCJ3aWR0aCI6MzcuMDg5Mjg2ODA0MTk5MjIsImhlaWdodCI6MjYuMDg4NjQwMjEzMDEyNjk1fX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNTg5In19XQ%3D%3D. Early warning systems via citizen science and toxin monitoring are crucial, as bloom collapses unpredictably elevate dissolved toxins (Haxthausen, 2021)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkthcmwgUmFzbXVzIEF1Z3VzdCIsImZhbWlseSI6IkhheHRoYXVzZW4ifX1dLCJpZHMiOnsiaXNzbiI6W10sImlzYm4iOltdLCJtYWciOiIzMTc3MTEzMjA3Iiwib3BlbmFsZXgiOiJXMzE3NzExMzIwNyJ9LCJ0aXRsZSI6IlNlZGltZW50IG1pY3JvYmlhbCBmdWVsIGNlbGxzIGZvciBsYWtlIHJlc3RvcmF0aW9uIiwiYWJzdHJhY3QiOiJFdXRyb3BoaWNhdGlvbiBvZiBsYWtlcyBpcyB1YmlxdWl0b3VzLCBhcyAzMC00MCAlIG9mIHRoZSB3b3JsZOKAmXMgbGFrZXMgYXJlIGV1dHJvcGhpYywgYW5kIGNvbmRpdGlvbnMgY29udGludW91c2x5IGRldGVyaW9yYXRpbmcuIEV1dHJvcGhpY2F0aW9uIGlzIGEgY29uZGl0aW9uIG9mIG92ZXJmZXJ0aWxpemF0aW9uLCB3aGVyZSBhbGwgcmVxdWlyZW1lbnRzIGZvciByYXBpZCBncm93dGggb2YgYWxnYWUgYXJlIG1ldCwgcmVzdWx0aW5nIGluIGFsZ2FsIGJsb29tcy4gVGhpcyBjYXVzZXMgYSBzZWxmLXN1c3RhaW5lZCwgdmljaW91cyBjeWNsZSB3aGVyZSBwb29yIGVjb2xvZ2ljYWwgY29uZGl0aW9ucyBhcmUgbWFpbnRhaW5lZCB1bnRpbCBudXRyaWVudHMgYXJlIGxvc3QgZnJvbSB0aGUgbGFrZSBvdmVyIG1hbnkgeWVhcnMuIExha2UgcmVzdG9yYXRpb24gc2Vla3MgdG8gYnJlYWsgdGhpcyB2aWNpb3VzIGN5Y2xlLiBJbmFkZXF1YXRlIGF2YWlsYWJpbGl0eSBvZiBhbnkgc2luZ2xlIG51dHJpZW50IGluIHdpbGwgbGltaXQgYWxnYWwgZ3Jvd3RoLCB0eXBpY2FsbHkgcGhvc3Bob3J1cyBpcyB0YXJnZXRlZC4gVGhlIHNjb3BlIG9mIHRoaXMgUGhEIHRoZXNpcyB3YXMgdG8gZGV2ZWxvcCwgdGVzdCBhbmQgb3B0aW1pemUgc2VkaW1lbnQgbWljcm9iaWFsIGZ1ZWwgY2VsbHMgKFNNRkMpIGFzIGEgbm92ZWwgbWV0aG9kIGZvciBsYWtlIHJlc3RvcmF0aW9uLiBUaGUgdGhlc2lzIHNwYW5zIHRoZSB0d28gbWFpbiB0b3BpY3Mgb2YgU01GQ3MgYW5kIGxha2UgcmVzdG9yYXRpb24gdGVjaG5pcXVlcy4gTGFrZSByZXN0b3JhdGlvbiB0ZWNobmlxdWVzIGFyZSByZXZpZXdlZCwgcHJvdmlkaW5nIHVuZGVyc3RhbmRpbmcgb2YgdGhlIHVuZGVybHlpbmcgbWVjaGFuaXNtcy4gU01GQ3MgYXJlIGFwcHJvYWNoZWQgZnJvbSBhbiBlbmdpbmVlcmluZyBwZXJzcGVjdGl2ZSBhbmQgYSBmb2N1cyBvbiBsYWtlIHJlbWVkaWF0aW9uLiBUaGUgc3BlY2lmaWMgb2JqZWN0aXZlcyB3ZXJlIHRvIHRlc3QgYSBsb3ctY29zdCBlbGVjdHJvZGUgbWF0ZXJpYWwgZm9yIGl0cyBkdXJhYmlsaXR5LCBwcmFjdGljYWxpdHksIGFuZCBwZXJmb3JtYW5jZS4gUGVyZm9ybWFuY2Ugd2FzIGFzc2Vzc2VkIGFzIHRoZSBhYmlsaXR5IHRvIGxpbWl0IHBob3NwaG9ydXMgcmVsZWFzZSBmcm9tIHNlZGltZW50cywgaW5oaWJpdCBtZXRoYW5lIHJlbGVhc2UsIGFuZCB0aGUgYWRkZWQgYmVuZWZpdHMgb2YgZGVncmFkaW5nIHBvbGx1dGFudHMsIGFuZCBnZW5lcmF0aW5nIGVsZWN0cmljaXR5LiBQcm9vZiBvZiBjb25jZXB0IHdhcyBzaG93biBpbiBsYWJvcmF0b3J5IHJlYWN0b3JzICgxIEwpLCB0aGUgZWZmZWN0IG9mIHVwc2NhbGluZyB0byAyMDAgTCBpbiBuYXR1cmFsIGVudmlyb25tZW50IHdhcyBzdHVkaWVkIGluIG91dGRvb3IgbWVzb2Nvc21zLCBhbmQgdGhlbiB0ZXN0ZWQgYXQgcGlsb3Qgc2NhbGUgaW4gYSBldXRyb3BoaWMgZnJlc2h3YXRlciBsYWtlLiBQaWxvdCBpbnN0YWxsYXRpb25zIHdlcmUgdGhlIGxhcmdlc3QgaW4gdGhlIHdvcmxkIG9mIHRoZWlyIGtpbmQgYW5kIHdlcmUgb3BlcmF0ZWQgZm9yIDIgeWVhcnMuIFN0YWlubGVzcyBzdGVlbCB3YXMgc2VsZWN0ZWQgZm9yIHRlc3RpbmcgYXMgZWxlY3Ryb2RlIG1hdGVyaWFsIGR1ZSB0byBpdHMgbG93IGNvc3QsIGR1cmFiaWxpdHksIGFuZCBhdmFpbGFiaWxpdHkuIFBhaXJzIG9mIGVsZWN0cm9kZXMgd2VyZSBlbWJlZGRlZCBpbiB0aGUgc2VkaW1lbnQgKHRoaXMgaXMgY2FsbGVkIHRoZSBhbm9kZSkgYW5kIHN1c3BlbmRlZCBpbiB0aGUgb3Zlcmx5aW5nIHdhdGVyICh0aGlzIGlzIGNhbGxlZCB0aGUgY2F0aG9kZSkuIFRoZSB0d28gZWxlY3Ryb2RlcyB3ZXJlIGNvbm5lY3RlZCBhY3Jvc3MgYW4gZXh0ZXJuYWwgcmVzaXN0b3Igd2l0aCBlbGVjdHJpY2FsIHdpcmVzLiBNaWNyb2JlcyBpbiB0aGUgc2VkaW1lbnQgdXNlIHRoZSBhbm9kZSBwYWlyIGFzIGVsZWN0cm9uIGFjY2VwdG9yLCBlbmFibGluZyByZXNwaXJhdGlvbiBpbiB0aGUgYW5veGljIHNlZGltZW50LiBFbGVjdHJvbnMgdHJhdmVsIGZyb20gdGhlIHNlZGltZW50IHZpYSB0aGUgZWxlY3RyaWNhbCBjb25uZWN0b3IgdG8gdGhlIG92ZXJseWluZyBjYXRob2RlIHdoZXJlIG94eWdlbiBpbiB0aGUgd2F0ZXIgaXMgdGhlIHRlcm1pbmFsIGVsZWN0cm9uIGFjY2VwdG9yIGNvbXBsZXRpbmcgdGhlIGNpcmN1aXQuIEFzIGFuIGFkZGVkIGJlbmVmaXQsIGRlZ3JhZGF0aW9uIG9mIHBvbGx1dGFudHMgdXNpbmcgU01GQyB3YXMgdGVzdGVkIGluIG1hcmluZSBoYXJib3VyIHNlZGltZW50cywgYnV0IG5vIGNvbmNsdXNpdmUgcmVzdWx0cyBvYnRhaW5lZC4gTGFib3JhdG9yeSByZWFjdG9ycyB3ZXJlIG9wZXJhdGVkIGZvciBtb3JlIHRoYW4gMjAwIGRheXMgdXNpbmcgcGV0cm9sZXVtIGFzIHNvbGUgY2FyYm9uIHNvdXJjZSwgY29uZmlybWluZyBvaWwgZGVncmFkYXRpb24gYnkgU01GQ3MuIEEgcmVtb3ZhbCByYXRlIG9mIDAuOSBnIHBldHJvbGV1bSBtLTMgZC0xIHJlYWN0b3Igdm9sdW1lIHdhcyBlc3RpbWF0ZWQgZnJvbSBtZWFzdXJlZCBlbGVjdHJpY2FsIG91dHB1dC4gVGhpcyB3YXMgZG9uZSBieSBhc3N1bXB0aW9uIG9mIHRoZSBjaGVtaWNhbCByZWFjdGlvbiBvY2N1cnJpbmcsIGZyb20gdGhpcyB0aGUgbnVtYmVyIG9mIGVsZWN0cm9ucyB0cmFuc2ZlcnJlZCBmcm9tIG9pbCB0byB3YXRlciB3YXMga25vd24gYW5kIGNvdWxkIGJlIGNvcnJlbGF0ZWQgdG8gdGhlIG1lYXN1cmVkIGZsb3cgb2YgZWxlY3Ryb25zIChjdXJyZW50KSB0aHJvdWdoIHRoZSBleHRlcm5hbCBjaXJjdWl0LiBTdGFpbmxlc3Mgc3RlZWwgZWxlY3Ryb2RlcyB3ZXJlIG9wZXJhdGVkIGluIGZyZXNod2F0ZXIgbGFrZSBlbnZpcm9ubWVudHMgZm9yIG1vcmUgdGhhbiA0IHllYXJzIHdpdGggbWluaW1hbCBkYW1hZ2UgdG8gdGhlIG1hdGVyaWFsLiBUaGUgZWxlY3Ryb2RlIGlzIHRoZSBtYWluIG1hdGVyaWFsIGNvc3QgaW4gc3VjaCBzeXN0ZW1zLiBIb3dldmVyLCBBSVNJMzE2IHN0YWlubGVzcyBzdGVlbCBzaW1pbGFyIHRvIHdoYXQgd2FzIHVzZWQgaW4gdGhpcyB0aGVzaXMgd2FzIGF0IHRoZSB0aW1lIG9mIHdyaXRpbmcgYXZhaWxhYmxlIGF0IGJ1bGsgY29zdCBvZiAwLjA5MSDigqwgbS0yLCBjb3JyZXNwb25kaW5nIHRvIDkxMCDigqwgYWNyZS0xLCBzaWduaWZpY2FudGx5IGxvd2VyIHRoYW4gb3RoZXIgY29tbW9uIG1hdGVyaWFscy4gQ2F0aG9kZXMgYXQgbGFyZ2Ugc2NhbGUgY291bGQgYmUgbW92ZWQgaG9yaXpvbnRhbGx5IGFuZCB2ZXJ0aWNhbGx5IHdpdGggbWlub3IgaW1wYWN0IHRvIHBlcmZvcm1hbmNlLiBQaWxvdCBzY2FsZSBTTUZDcyBnZW5lcmF0ZWQgMC4zODMgbVcgbS0yIHN1c3RhaW5lZCBhbmQgMjcuNCBtVyBtLTIgcGVhayBvdXRwdXQuIFRoZSBzeXN0ZW0gd2FzIGxpbWl0ZWQgYnkgdGhlIGNhdGhvZGUgc3VyZmFjZSBhcmVhLCBtZWFuaW5nIG11bHRpLWxheWVyIGNhdGhvZGVzIHNob3VsZCBiZSB0ZXN0ZWQgaW4gdGhlIGZ1dHVyZSBpZiBlbGVjdHJpY2FsIHBvd2VyIHByb2R1Y3Rpb24gaXMgdGhlIGdvYWwuIFRoZSBjYXRob2RlIHNob3VsZCBiZSBwbGFjZWQgYXQgYSBkZXB0aCB0aGF0IGVuc3VyZXMgaGlnaCBkaXNzb2x2ZWQgb3h5Z2VuIGNvbmNlbnRyYXRpb25zIHRvIGF2b2lkIHRoZSBzeXN0ZW0gdG8gYmUgbGltaXRlZC4gSG93ZXZlciwgdG9vIGhpZ2ggZWxlY3RyaWNpdHkgZ2VuZXJhdGlvbiB3YXMgZm91bmQgY291bnRlcnByb2R1Y3RpdmUgZm9yIHJlbWVkaWF0aW9uIHB1cnBvc2VzIGJlY2F1c2UgdGhpcyB3b3VsZCBpbmNyZWFzZSBzZWRpbWVudCB0dXJub3ZlciByYXRlLCByZWxlYXNpbmcgcG9sbHV0YW50cyBzdG9yZWQgaW4gdGhlIHNlZGltZW50cy4gUCBiaW5kaW5nIHdhcyBub3QgZm91bmQgdG8gcmVxdWlyZSBoaWdoIHR1cm5vdmVyIG9mIHNlZGltZW50IG9yZ2FuaWMgbWF0dGVyLiBUaGUgdHVybm92ZXIgY2FuIGJlIGNvbnRyb2xsZWQgYnkgY2hhbmdpbmcgdGhlIGV4dGVybmFsIHJlc2lzdGFuY2UuIEhpZ2ggcmVzaXN0YW5jZSBsaW1pdHMgdGhlIGVsZWN0cmljYWwgcGVyZm9ybWFuY2UsIGxvdyByZXNpc3RhbmNlIGluY3JlYXNlcyBzZWRpbWVudCB0dXJub3ZlciBhbmQgcmlza3MgZGVzdGFiaWxpemluZyB0aGUgc3lzdGVtLiBUbyBhY2hpZXZlIHJlZHVjZWQgY29uY2VudHJhdGlvbnMgb2YgcGhvc3BoYXRlIGluIHRoZSB3YXRlciBjb2x1bW4sIGFub2RlcyBtdXN0IGJlIHBsYWNlZCBpbiB0aGUgc2VkaW1lbnQgc3VyZmFjZSBzaW5jZSBhIGZldyBtaWxsaW1ldHJlcyBvZiBzZWRpbWVudCBtYXkgY29udGFpbiBhbmQgcmVsZWFzZSBlbm91Z2ggUCB0byBzdXN0YWluIGV1dHJvcGhpY2F0aW9uLiBQaG9zcGhhdGUgd2FzIG1lYXN1cmVkIGluIHRoZSB3YXRlciBjb2x1bW4gbmVhciB0aGUgc2VkaW1lbnQsIGluIHN5c3RlbXMgd2l0aCBhbmQgd2l0aG91dCBlbGVjdHJvZGVzLiBQaG9zcGhhdGUgY29uY2VudHJhdGlvbnMgd2VyZSBzdWNjZXNzZnVsbHkgcmVkdWNlZCBieSB1cCB0byA5NSAlIGluIDItY2hhbWJlciBsYWJvcmF0b3J5IHJlYWN0b3JzLCBhbmQgYnkgdXAgdG8gOTggJSBpbiB0aGUgcGlsb3QgaW5zdGFsbGF0aW9uLiBJbiB0aGUgcGlsb3QgZXhwZXJpbWVudCwgcGhvc3BoYXRlIGNvbmNlbnRyYXRpb25zIGFib3ZlIGVsZWN0cm9kZXMgd2VyZSBsb3dlciBpbiBhbGwgbWVhc3VyZW1lbnRzLCBjb21wYXJlZCB0byBjb250cm9scy4gU2VkaW1lbnQgcmVkb3ggcG90ZW50aWFscyBpbmNyZWFzZWQgZnJvbSAtNDAwIG1WIHZzIHN0YW5kYXJkIGh5ZHJvZ2VuIGVsZWN0cm9kZSAoU0hFKSB0byAtMTc1IG1WIHZzIFNIRSwgY29ycmVzcG9uZGluZyB0byBhIG1vdmUgZnJvbSBhIG1ldGhhbm9nZW5ldGljIGRvbWFpbiB0byBzdWxwaHVyIHJlZHVjaW5nLiBBbmFseXNpcyBvZiB0aGUgbWljcm9iaWFsIGNvbW11bml0eSBpbmRpY2F0ZWQgZW5oYW5jZWQgc3VscGh1ciBjeWNsaW5nIHdoZXJlIGVsZWN0cm9kZXMgb3hpZGl6ZSBzdWxwaGlkZSB3aGljaCB0aGUgYmFjdGVyaWEgYWdhaW4gcmVkdWNlIHRvIG9idGFpbiBlbmVyZ3kuIEl0IHdhcyBoeXBvdGhlc2lzZWQgdGhhdCBTTUZDIHByb21vdGUgaGlnaGVyIHJlZG94IHBvdGVudGlhbHMgYnkgZmFjaWxpdGF0aW5nIG94aWRhdGlvbiBvZiBzdWxwaHVyIGFuZCBpcm9uIHNwZWNpZXMsIGluaGliaXRpbmcgSDJTIGZvcm1hdGlvbiBhbmQgRmUgc2NhdmVuZ2luZyBieSBGZVMgZm9ybWF0aW9uLCB0aHVzIGFsbG93aW5nIGZvciBQIGJpbmRpbmcuIEEgbG93ZXIgcmVsYXRpdmUgYWJ1bmRhbmNlIG9mIG1ldGhhbm9nZW5zIG9uIGVsZWN0cm9kZXMgY29tcGFyZWQgdG8gY29udHJvbHMgc3VnZ2VzdHMgc3VwcHJlc3Npb24gb3Igb3V0Y29tcGV0aW5nIG9mIG1ldGhhbm9nZW5zLlF1YW50aWZpY2F0aW9uIG9mIG1ldGhhbmUgcHJvZHVjdGlvbiB3YXMgZG9uZSBvbiB0aGUgYmFzaXMgb2YgYSBDSDQ6Q08yIHJhdGlvIHRvIGFsbG93IGNvbXBhcmFiaWxpdHkgYmV0d2VlbiBzeXN0ZW1zLiBNZXRoYW5lIGVtaXNzaW9uIHdhcyByZWR1Y2VkIGJ5IHVwIHRvIDk5ICUgYW5kIG1haW50YWluZWQgaW5oaWJpdGlvbiBmb3Igc2V2ZXJhbCBtb250aHMgaW4gbGFib3JhdG9yeSBzeXN0ZW1zIHdpdGggZWxlY3Ryb2Rlcy4gSW4gdGhlIHBpbG90IGluc3RhbGxhdGlvbnMgbm8gZ2FzIHNhbXBsZXMgY291bGQgYmUgY29sbGVjdGVkLCBhbmQgbm8gQ08yOkNINCByYXRpb3Mgd2VyZSBvYnRhaW5lZC4gQWVyYXRpb24gaXMgdGhlIGN1cnJlbnQgcmVzdG9yYXRpb24gdGVjaG5pcXVlIHRoYXQgbW9zdCBjbG9zZWx5IHJlc2VtYmxlcyB0aGUgcHJvcG9zZWQgU01GQyB0ZWNobmlxdWUuIEFlcmF0aW9uIGhhcyBwcmV2aW91c2x5IGZhaWxlZCB0byByZWR1Y2UgUCByZWxlYXNlIGluIHNldmVyYWwgc2VwYXJhdGUgaW52ZXN0aWdhdGlvbnMuIFNNRkNzIGFzIGEgcmVtZWRpYXRpb24gdG9vbCB3YXMgZm91bmQgdG8gb2ZmZXIgc2V2ZXJhbCBiZW5lZml0cyBvdmVyIGNvbnZlbnRpb25hbCBtZXRob2RzOiBTTUZDcyBvZmZlcnMgY29udGludW91cyBlZmZlY3QsIHdoaWxlIG90aGVyIHRlY2huaXF1ZXMgcmVxdWlyZSBjb250aW51b3VzIG9yIHJlcGVhdGVkIHRyZWF0bWVudC4gU01GQ3MgY2FuIGJlIGNvbnRyb2xsZWQgaW4gcmVhbC10aW1lIGJ5IGFsdGVyaW5nIHRoZSBleHRlcm5hbCByZXNpc3RvciBhbmQgY2FuIGJlIGVhc2lseSBtb25pdG9yZWQgYnkgbWVhc3VyaW5nIHRoZSBwcm9kdWNlZCBlbGVjdHJpY2FsIG91dHB1dC4gSWYgaW1wbGVtZW50ZWQgYXQgbGFyZ2Ugc2NhbGUgaXQgaXMgZXhwZWN0ZWQgdGhhdCB0aGUgZWxlY3Ryb2RlIGZpZWxkcyB3aWxsIGhlbHAgbWFpbnRhaW4gbG93IHBob3NwaGF0ZSBjb25jZW50cmF0aW9ucyBpbiB0aGUgd2F0ZXIgZm9yIGRlY2FkZXMgd2l0aCBtaW5pbWFsIGludGVydmVudGlvbiwgYW5kIGFmdGVyIHNvbWUgdGltZSwgYmVudGhpYyBwbGFudHMgYXJlIGV4cGVjdGVkIHRvIHJldHVybiwgdGh1cyBtYWludGFpbmluZyB0aGUgYmFsYW5jZSBoZW5jZSBmb3J0aC4iLCJ1cmwiOiJodHRwczovL2xvY2FsLmZvcnNrbmluZ3Nwb3J0YWwuZGsvbG9jYWwvZGtpLWNnaS93cy9jcmlzLWxpbms%2Fc3JjPWR0dSZpZD1kdHUtZDY2ZDMzNzAtMjU2Zi00ODg1LWEwOWUtMzM2YTkyNGY0YmVkJnRpPVNlZGltZW50JTIwbWljcm9iaWFsJTIwZnVlbCUyMGNlbGxzJTIwZm9yJTIwbGFrZSUyMHJlc3RvcmF0aW9uIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2JhY2tlbmQub3JiaXQuZHR1LmRrL3dzL2ZpbGVzLzI0ODA2OTU4OS9UaGVzaXNfb25saW5lX3ZlcnNpb25fS2FybF9IYXh0aGF1c2VuLnBkZiIsImlzc3VlZERhdGUiOiIyMDIxLTAxLTAxIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0wOCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiUmVzZWFyY2ggUG9ydGFsIERlbm1hcmsiLCJpbXBhY3RGYWN0b3IiOiIwLjA0In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6NzZ9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiVGVjaG5pY2FsIFVuaXZlcnNpdHkgb2YgRGVubWFyayJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MCwiY2l0ZWRCeUNvdW50IjoxfX0sInF1b3RlIjoiRXh0ZXJuYWwgaW5wdXRzIGluY3JlYXNlIFAgY29uY2VudHJhdGlvbiBpbiAgdGhlIHdhdGVyLCBjYXVzaW5nIGFsZ2FsIGJsb29tcyB0aGF0IHNoYWRvdyB0aGUgbGFrZSBib3R0b20sIGFuZCBiZW50aGljIG1hYy1yb3BoeXRlcyByZWNlZGUuIFByZWRhdG9yeSBmaXNoIGFyZSB1bmFibGUgdG8gaHVudCwgYW5kIHRoZWlyIGZvb2Qgc291cmNlIGlzICBkaXNydXB0ZWQgc3VjaCBhcyBmaXNoIGZlZWRpbmcgb24gYmVudGhpYyBpbnZlcnRlYnJhdGVzIGFuZCBib3R0b20tZHdlbGxpbmcgYW5pbWFscyBhcmUgc3VmZm9jYXRlZCAoVmFuZGVyIFphbmRlbiBhbmQgVmFkZWJvbmNvZXVyIDIwMDIpLCBzbyB0aGF0IHBsYW5rdGl2b3JvdXMgZmlzaCBzcGVjaWVzIGRvbWluYXRlLiBUaGlzIGluY3JlYXNlcyBwcmVkYXRpb24gb24gem9vIHBsYW5rdG9uICBhbmQgcmVkdWNlcyBncmF6aW5nIG9uIGFsZ2FlLiBBbGdhZSBwcm9saWZlcmF0ZSwgZGllLCBhbmQgc2luayB0byB0aGUgYm90dG9tLCAgY29uc3VtaW5nIG94eWdlbiB3aGVuIGRlZ3JhZGVkLCB3aGljaCBsZWFkcyB0byBveHlnZW4gZGVmaWNpdCBpbiB0aGUgYm90LXRvbSwga2lsbGluZyBmaXNoIChXdXJ0c2JhdWdoIGV0IGFsLiAyMDE5KS4gRmlndXJlIDI6IFZpY2lvdXMgY3ljbGVzIG9mIGV1dHJvcGhpY2F0aW9uLiBMZWZ0OiBJbnRlcmxpbmtlZCBwcm9jZXNzZXMgc3VzdGFpbmluZyBldXRyb3BoaS1jYXRpb24gaW4gYSBsYWtlLiBCbHVlOiBwcm9jZXNzZXMgaW4gd2F0ZXIsIGdyZWVuOiBwcm9jZXNzZXMgaW52b2x2aW5nIG1pY3JvYWxnYWUsIGdyZXk6IHNlZC1pbWVudCByZWxhdGVkIHByb2Nlc3NlcywgeWVsbG93OiBwcm9jZXNzZXMgaW52b2x2aW5nIGZhdW5hLiBSaWdodCwgZGFzaGVkIGxpbmU6IFRoZSBvcmRlciBpbiB3aGljaCBlbGVjdHJvbiBhY2NlcHRvcnMgYXJlIGNvbnN1bWVkIGluIHRoZSBhYnNlbmNlIG9mIE8yIHNob3duIGFzIGNvbmNlbnRyYXRpb24gdmVyc3VzICBkZXB0aCBpbiBzZWRpbWVudCwgYW5kIGF0IHdoaWNoIHBvdGVudGlhbCB0aGlzIG9jY3VycyAoQmVybmVyIDE5ODEsIGJ5IGNvdXJ0ZXN5IG9mIHRoZSBzb2NpZXR5ICBmb3Igc2VkaW1lbnRhcnkgZ2VvbG9neSkuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxNywiYmJveCI6eyJ0b3AiOjc5LjkzMzE1MTI0NTExNzE5LCJsZWZ0IjoxNC4yOTU1MDM2MTYzMzMwMDgsIndpZHRoIjo3MS45Mzg5MzQzMjYxNzE4OCwiaGVpZ2h0IjoyLjA1MzQ3NzI4NzI5MjQ4MDV9fSx7InBhZ2VOdW1iZXIiOjE3LCJiYm94Ijp7InRvcCI6ODEuOTcxMzQzOTk0MTQwNjIsImxlZnQiOjE0LjI5NTUwMzYxNjMzMzAwOCwid2lkdGgiOjcxLjMzMzAwMDE4MzEwNTQ3LCJoZWlnaHQiOjIuMDUzNDc3Mjg3MjkyNDgwNX19LHsicGFnZU51bWJlciI6MTcsImJib3giOnsidG9wIjo4NC4wMjM3OTYwODE1NDI5NywibGVmdCI6MTQuMjk1NTAzNjE2MzMzMDA4LCJ3aWR0aCI6NzEuODEyMDE5MzQ4MTQ0NTMsImhlaWdodCI6Mi4wNTM0NzcyODcyOTI0ODA1fX0seyJwYWdlTnVtYmVyIjoxNywiYmJveCI6eyJ0b3AiOjg2LjA2MTk5NjQ1OTk2MDk0LCJsZWZ0IjoxNC4yOTU1MDM2MTYzMzMwMDgsIndpZHRoIjo3MS45NTczNTkzMTM5NjQ4NCwiaGVpZ2h0IjoyLjA1MzQ3NzI4NzI5MjQ4MDV9fSx7InBhZ2VOdW1iZXIiOjE4LCJiYm94Ijp7InRvcCI6OS45MjY0ODc5MjI2Njg0NTcsImxlZnQiOjE0LjI5NTUwMzYxNjMzMzAwOCwid2lkdGgiOjcxLjk3NDA2MDA1ODU5Mzc1LCJoZWlnaHQiOjEwLjIzNDc3NzQ1MDU2MTUyM319LHsicGFnZU51bWJlciI6MTgsImJib3giOnsidG9wIjo0NC43MzQwNDMxMjEzMzc4OSwibGVmdCI6MTQuMjk1NTAzNjE2MzMzMDA4LCJ3aWR0aCI6NzEuODkzNDMyNjE3MTg3NSwiaGVpZ2h0IjoxMC40MDMyODk3OTQ5MjE4NzV9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI5MyJ9fV0%3D. Projections under warming scenarios predict 2-3 fold toxin increases by 2050, necessitating regulatory toxin thresholds, livestock advisories, and public awareness campaigns (Jansen et al., 2024; Nõges et al., 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkpvYWNoaW0iLCJmYW1pbHkiOiJKYW5zZW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiR2F2aW4gTC4iLCJmYW1pbHkiOiJTaW1wc29uIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikdlc2EgQS4iLCJmYW1pbHkiOiJXZXloZW5tZXllciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJMYXVyYSBILiIsImZhbWlseSI6IkjDpHJrw7ZuZW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW5kcmV3IE0uIiwiZmFtaWx5IjoiUGF0ZXJzb24ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUGF1bCBBLiBkZWwiLCJmYW1pbHkiOiJHaW9yZ2lvIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ill2ZXMgVC4iLCJmYW1pbHkiOiJQcmFpcmllIn19XSwiaWRzIjp7Imlzc24iOlsiMTc1OC02NzhYIiwiMTc1OC02Nzk4Il0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDM4L3M0MTU1OC0wMjQtMDIwNTgtMyIsIm9wZW5hbGV4IjoiVzQ0MDAyMDgyMDAiLCJleGEiOiJodHRwczovL3d3dy5uYXR1cmUuY29tL2FydGljbGVzL3M0MTU1OC0wMjQtMDIwNTgtMz9lcnJvcj1jb29raWVzX25vdF9zdXBwb3J0ZWQmY29kZT1hNzRhMjVmZi1hNThkLTRkZjgtOTM2Zi0yNjJlMmUzZTkxOTgifSwidGl0bGUiOiJDbGltYXRlLWRyaXZlbiBkZW94eWdlbmF0aW9uIG9mIG5vcnRoZXJuIGxha2VzIiwiYWJzdHJhY3QiOiJBYnN0cmFjdCBPeHlnZW4gZGVwbGV0aW9uIGNvbnN0aXR1dGVzIGEgbWFqb3IgdGhyZWF0IHRvIGxha2UgZWNvc3lzdGVtcyBhbmQgdGhlIHNlcnZpY2VzIHRoZXkgcHJvdmlkZS4gTW9zdCBvZiB0aGUgd29ybGTigJlzIGxha2VzIGFyZSBsb2NhdGVkICZndDs0NcKwIE4sIHdoZXJlIGFjY2VsZXJhdGVkIGNsaW1hdGUgd2FybWluZyBhbmQgZWxldmF0ZWQgY2FyYm9uIGxvYWRzIG1pZ2h0IHNldmVyZWx5IGluY3JlYXNlIHRoZSByaXNrIG9mIGh5cG94aWEsIGJ1dCB0aGlzIGhhcyBub3QgYmVlbiBzeXN0ZW1hdGljYWxseSBleGFtaW5lZC4gSGVyZSBhbmFseXNpcyBvZiAyLjYgbWlsbGlvbiB3YXRlciBxdWFsaXR5IG9ic2VydmF0aW9ucyBmcm9tIDgsMjg4IGxha2VzIHNob3dzIHRoYXQgYmV0d2VlbiAxOTYwIGFuZCAyMDIyLCBtb3N0IG5vcnRoZXJuIGxha2VzIGV4cGVyaWVuY2VkIHJhcGlkIGRlb3h5Z2VuYXRpb24gc3Ryb25nbHkgbGlua2VkIHRvIGNsaW1hdGUtZHJpdmVuIHByb2xvbmdhdGlvbiBvZiBzdW1tZXIgc3RyYXRpZmljYXRpb24uIE94eWdlbiBsZXZlbHMgZGV0ZXJpb3JhdGVkIG1vc3QgaW4gc21hbGwgbGFrZXMgKCZsdDsxMCBoYSkgb3dpbmcgdG8gdGhlaXIgZ3JlYXRlciB2b2x1bWV0cmljIG94eWdlbiBkZW1hbmQgYW5kIHN1cmZhY2Ugd2FybWluZyByYXRlcywgd2hpbGUgdGhlIGxhcmdlc3QgbGFrZXMgZ2FpbmVkIG94eWdlbiB1bmRlciBtaW5pbWFsIHN0cmF0aWZpY2F0aW9uIGNoYW5nZXMgYW5kIGltcHJvdmVkIGFlcmF0aW9uIGF0IHNwcmluZyBvdmVydHVybnMuIFNlYXNvbmFsIG94eWdlbiBjb25zdW1wdGlvbiByYXRlcyBkZWNsaW5lZCwgZGVzcGl0ZSB3aWRlc3ByZWFkIGJyb3duaW5nLiBQcm9saWZlcmF0aW5nIGFub3hpYSBlbmhhbmNlZCBzZWFzb25hbCBpbnRlcm5hbCBsb2FkaW5nIG9mIEMsIFAgYW5kIE4gYnV0IGRlcGxldGVkIFAgbG9uZy10ZXJtLCBpbmRpY2F0aW5nIHRoYXQgZGVveHlnZW5hdGlvbiBjYW4gZXhoYXVzdCByZWRveC1zZW5zaXRpdmUgZnJhY3Rpb25zIG9mIHNlZGltZW50IG51dHJpZW50IHJlc2Vydm9pcnMuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjEwMzgvczQxNTU4LTAyNC0wMjA1OC0zIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAzOC9zNDE1NTgtMDI0LTAyMDU4LTMiLCJpc3N1ZWREYXRlIjoiMjAyNC0wNy0wMSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTEiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6Ik5hdHVyZSBDbGltYXRlIENoYW5nZSIsImlzc3VlIjo4LCJ2b2x1bWUiOjE0LCJpbXBhY3RGYWN0b3IiOiIxNS4wMiJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjgzMiwibGFzdFBhZ2UiOjgzOH19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJOYXR1cmUgUG9ydGZvbGlvIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo4MCwiY2l0ZWRCeUNvdW50IjozN319LCJxdW90ZSI6IkFlcm9iaWMgcmVzcGlyYXRpb24gY2FuIGRlcGxldGUgb3h5Z2VuIGNvbmNlbnRyYXRpb25zIGF0IGRlcHRoIHdoZW4gc2Vhc29uYWwgc3RyYXRpZmljYXRpb24gcHJldmVudHMgYWVyYXRpb24gb2YgdGhlIHdhdGVyIGNvbHVtbls1XShodHRwczovL3d3dy5uYXR1cmUuY29tL2FydGljbGVzL3M0MTU1OC0wMjQtMDIwNTgtMyNyZWYtQ1I1KS4gVGhpcyBkZXRyaW1lbnRhbCBjb25kaXRpb24sIHRlcm1lZCBoeXBveGlhIChET%2BKAiTzigIky4oCJbWfigIls4oiSMSksIGRpbWluaXNoZXMgdGhlIGNvbGQtd2F0ZXIgaGFiaXRhdCBvZiBhcXVhdGljIGxpZmUsIGluY2x1ZGluZyBjb21tZXJjaWFsbHkgaW1wb3J0YW50IGZpc2hbNl0oaHR0cHM6Ly93d3cubmF0dXJlLmNvbS9hcnRpY2xlcy9zNDE1NTgtMDI0LTAyMDU4LTMjcmVmLUNSNikgYW5kIGNhbiBjYXVzZSBtYXNzIG1vcnRhbGl0eSBpZiBwZXJzaXN0ZW50WzddKGh0dHBzOi8vd3d3Lm5hdHVyZS5jb20vYXJ0aWNsZXMvczQxNTU4LTAyNC0wMjA1OC0zI3JlZi1DUjcpLiBBbm94aWEgKERP4oCJPOKAiTAuNeKAiW1n4oCJbOKIkjEpIHByb21vdGVzIHRoZSByZWR1Y3RpdmUgZGlzc29sdXRpb24gb2YgbWluZXJhbC1ib3VuZCBjYXJib24gYW5kIHBob3NwaG9ydXNbOF0oaHR0cHM6Ly93d3cubmF0dXJlLmNvbS9hcnRpY2xlcy9zNDE1NTgtMDI0LTAyMDU4LTMjcmVmLUNSOCksIFs5XShodHRwczovL3d3dy5uYXR1cmUuY29tL2FydGljbGVzL3M0MTU1OC0wMjQtMDIwNTgtMyNyZWYtQ1I5KSB3aGljaCBmdWVscyBoYXJtZnVsIGFsZ2FsIGJsb29tc1sxMF0oaHR0cHM6Ly93d3cubmF0dXJlLmNvbS9hcnRpY2xlcy9zNDE1NTgtMDI0LTAyMDU4LTMjcmVmLUNSMTApIGFuZCBsaW1pdHMgbWljcm9iaWFsIG94aWRhdGlvbiBvZiBtZXRoYW5lLCBhIHBvdGVudCBncmVlbmhvdXNlIGdhc1sxMV0oaHR0cHM6Ly93d3cubmF0dXJlLmNvbS9hcnRpY2xlcy9zNDE1NTgtMDI0LTAyMDU4LTMjcmVmLUNSMTEpLiJ9LHsib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRpaW5hIiwiZmFtaWx5IjoiTsO1Z2VzIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hc2ltZSIsImZhbWlseSI6IkphbmF0aWFuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlJlZXQiLCJmYW1pbHkiOiJMYXVnYXN0ZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQZWV0ZXIiLCJmYW1pbHkiOiJOw7VnZXMifX1dLCJpZHMiOnsiaXNzbiI6WyIyMzUxLTk4OTQiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMTYvai5nZWNjby4yMDIwLmUwMTM2OSIsIm1hZyI6IjMxMDU0NDYwOTAiLCJvcGVuYWxleCI6IlczMTA1NDQ2MDkwIiwiZXhhIjoiaHR0cHM6Ly93d3cuc2NpZW5jZWRpcmVjdC5jb20vc2NpZW5jZS9hcnRpY2xlL3BpaS9TMjM1MTk4OTQyMDMwOTEwMCJ9LCJ0aXRsZSI6IlBvc3Qtc292aWV0IGNoYW5nZXMgaW4gbml0cm9nZW4gYW5kIHBob3NwaG9ydXMgc3RvaWNoaW9tZXRyeSBpbiB0d28gbGFyZ2Ugbm9uLXN0cmF0aWZpZWQgbGFrZXMgYW5kIHRoZSBpbXBhY3Qgb24gcGh5dG9wbGFua3RvbiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2ouZ2VjY28uMjAyMC5lMDEzNjkiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2ouZ2VjY28uMjAyMC5lMDEzNjkiLCJpc3N1ZWREYXRlIjoiMjAyMC0xMS0yMCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTEiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6Ikdsb2JhbCBFY29sb2d5IGFuZCBDb25zZXJ2YXRpb24iLCJ2b2x1bWUiOjI0LCJpbXBhY3RGYWN0b3IiOiIzLjU5In0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkVsc2V2aWVyIEJWIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo3MSwiY2l0ZWRCeUNvdW50IjoxNH19LCJxdW90ZSI6IlJhbmRvbSBGb3Jlc3QgYW5hbHlzaXMgaW5kaWNhdGVkIGFuIGltcG9ydGFudCByb2xlIG9mIGxpZ2h0IGxpbWl0YXRpb24gaW4gYm90aCBsYWtlcy4gQ29uc3RhbnRseSBsb3dlciBsZXZlbHMgb2YgTiBpbiB0aGUgZGVlcGVyIExha2UgUGVpcHNpIGZhdm91cmVkIE4yLWZpeGluZyBzcGVjaWVzLCB3aGljaCwgYXMgYSBwYXJhZG94LCBiZWNhbWUgUC1saW1pdGVkLiBBcyBjbGltYXRlIHdhcm1pbmcgcmVpbmZvcmNlcyBldXRyb3BoaWNhdGlvbiBwaGVub21lbmEgaW4gbGFrZXMgYnkgaW5jcmVhc2luZyBpbnRlcm5hbCBudXRyaWVudCBsb2FkaW5nIGFuZCBmYXZvdXJpbmcgYmxvb20tZm9ybWluZyBjeWFub2JhY3RlcmlhLCBtb3JlIHN0cmluZ2VudCBtZWFzdXJlcyB3b3VsZCBiZSBuZWVkZWQgdG8gZnVydGhlciBsaW1pdCBudXRyaWVudCBsb2FkcyAoZXNwZWNpYWxseSB0aGF0IG9mIFApIHRvIGxha2VzIHRocm91Z2ggaW1wcm92ZWQgd2FzdGV3YXRlciB0cmVhdG1lbnQgYW5kIGluY3JlYXNlZCBlZmZpY2llbmN5IG9mIGZlcnRpbGlzZXIgYXBwbGljYXRpb24uIExvbmctdGVybSBjaGFuZ2VzIGluIG51dHJpZW50IGxvYWRpbmcgYW5kIHN0b2ljaGlvbWV0cnkgaW4gd2F0ZXIgYm9kaWVzIHJlZmxlY3QgY2hhbmdlcyBpbiBhZ3JpY3VsdHVyYWwgcHJhY3RpY2VzIGFuZCB3YXN0ZXdhdGVyIHRyZWF0bWVudC4ifV0%3D. By addressing nutrient legacies and climate feedbacks, communities can avert a public health crisis intertwined with ecological collapse.

9. Socio-Economic Impacts on Local Communities

Harmful algal blooms inflict severe financial tolls on inland fisheries that form the backbone of livelihoods for millions in India. In eutrophic lakes like Chilika and Loktak, cyanobacterial dominance and ensuing hypoxia decimate fish stocks, with mass mortalities reported during peak blooms (Jansen et al., 2024; Nair & Nayak, 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hdnlhIFZpa3JhbWFuIiwiZmFtaWx5IjoiTmFpciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQcmF0ZWVwIEt1bWFyIiwiZmFtaWx5IjoiTmF5YWsifX1dLCJpZHMiOnsiaXNzbiI6WyIyMjk2LTc3NDUiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzODkvZm1hcnMuMjAyMy4xMDg3Mjk2Iiwib3BlbmFsZXgiOiJXNDM4MjcyMzEzMyJ9LCJ0aXRsZSI6IlVuY292ZXJpbmcgd2F0ZXIgcXVhbGl0eSBhbmQgZXZhbHVhdGluZyB2dWxuZXJhYmlsaXRpZXMgb2Ygc21hbGwtc2NhbGUgZmlzaGVyaWVzIGluIENoaWxpa2EgTGFnb29uLCBJbmRpYSIsImFic3RyYWN0IjoiU21hbGwtc2NhbGUgZmlzaGVyaWVzIChTU0ZzKSBwbGF5IGEgc3Ryb25nIHJvbGUgaW4gc3VzdGFpbmluZyBtaWxsaW9ucyBvZiBsaXZlbGlob29kcywgZm9vZCBzZWN1cml0eSwgbnV0cml0aW9uLCBhbmQgaW5jb21lIGdsb2JhbGx5IGJ1dCB0aGUgZmlzaGVycyBlbmdhZ2VkIGluIHRoaXMgc2VjdG9yIHNpbXVsdGFuZW91c2x5IGV4cGVyaWVuY2UgaGlnaCBsZXZlbHMgb2YgdnVsbmVyYWJpbGl0eSBhbmQgcHJvY2Vzc2VzIG9mIG1hcmdpbmFsaXNhdGlvbi4gU2V2ZXJhbCBmYWN0b3JzIGFyZSBhdHRyaWJ1dGVkIHRvIHRoZSBtdWx0aWRpbWVuc2lvbmFsIHZ1bG5lcmFiaWxpdGllcyB0aGUgc21hbGwtc2NhbGUgZmlzaGVycyBleHBlcmllbmNlLCBzcGFubmluZyBib3RoIG5hdHVyYWwgKGUuZy4sIG5hdHVyYWwgZGlzYXN0ZXJzLCBlY29zeXN0ZW0gY2hhbmdlKSBhbmQgYW50aHJvcG9nZW5pYyAoZS5nLiwgcG9saWN5IGNoYW5nZSwgaHlkcm9sb2dpY2FsIGludGVydmVudGlvbnMsIGFxdWFjdWx0dXJlKSBwcmVzc3VyZXMuIFdoaWxlIHRoZXJlIGlzIG11Y2ggbGl0ZXJhdHVyZSBvbiB2YXJpb3VzIG5hdHVyYWwgYW5kIGh1bWFuIGRyaXZlcnMgb2YgdnVsbmVyYWJpbGl0eSBpbiBzbWFsbC1zY2FsZSBmaXNoZXJ5IGNvbW11bml0aWVzLCBhbiBhYnNlbmNlIG9mIHJlc2VhcmNoIGNvbm5lY3RpbmcgdnVsbmVyYWJpbGl0eSB3aXRoIHdhdGVyIHF1YWxpdHkgaXMgZXZpZGVudC4gRmlzaGVyIGNvbW11bml0aWVzIG9mdGVuIHRhbGsgYWJvdXQgZmlzaCBpbiByZWxhdGlvbiB0byB0aGUgaGVhbHRoIG9mIHRoZWlyIGFxdWF0aWMgaGFiaXRhdHMgd2hlcmVpbiB3YXRlciBxdWFsaXR5IGlzIHNlZW4gYXMgYSBrZXkgcGFyYW1ldGVyLiBUaGUgbGluayBiZXR3ZWVuIGhlYWx0aHkgZmlzaCBhbmQgZ29vZCBxdWFsaXR5IHdhdGVyIGhhcyBzaWduaWZpY2FudCBpbXBsaWNhdGlvbnMgZm9yIHN0cm9uZyBhbmQgdmlhYmxlIGZpc2hpbmcgY29tbXVuaXRpZXMuIFRoaXMgcGFwZXIgZXhhbWluZXMgdGhlc2UgbGlua3MgZnVydGhlciBieSBmb2N1c2luZyBvbiB0aGUgbmF0dXJlIG9mIHZ1bG5lcmFiaWxpdGllcyBjYXVzZWQgYnkgd2F0ZXIgcXVhbGl0eSBjaGFuZ2VzIGluIHRoZSBzbWFsbC1zY2FsZSBmaXNoZXJ5IHN5c3RlbSBvZiBDaGlsaWthIExhZ29vbiBpbiBJbmRpYS4gV2UgdW5kZXJ0YWtlIGRldGFpbGVkIGFuYWx5c2lzIG9mIHRoZSBpbnZhc2l2ZSBzaHJpbXAgYXF1YWN1bHR1cmUgYWN0aXZpdGllcyBhbmQgaHlkcm9sb2dpY2FsIGludGVydmVudGlvbnMgZm9yIG9wZW5pbmcgb2YgYSBsYWdvb24gaW5sZXQgd2l0aCB0aGUgQmF5IG9mIEJlbmdhbCBhcyB0d28gZG9taW5hbnQgZHJpdmVycyBhZHZlcnNlbHkgaW1wYWN0aW5nIHdhdGVyIHF1YWxpdHkgYW5kIGluY3JlYXNpbmcgdnVsbmVyYWJpbGl0aWVzIG9mIHRoZSBlbnRpcmUgc21hbGwtc2NhbGUgZmlzaGVyaWVzIHNvY2lhbC1lY29sb2dpY2FsIHN5c3RlbS4gT3VyIGFuYWx5c2lzIHN1Z2dlc3RzIHRoYXQgdGhlcmUgYXJlIHN0cm9uZyBpbnRlcmNvbm5lY3Rpb25zIGJldHdlZW4gY2hhbmdlcyBpbiB3YXRlciBxdWFsaXR5IGFuZCB0aGUgbGV2ZWxzIG9mIHZ1bG5lcmFiaWxpdGllcyBpbiB0aGUgU1NGcyBvZiBDaGlsaWthIExhZ29vbi4gUG9sbHV0YW50cyBzdWNoIGFzIHBlc3RpY2lkZXMsIGFuZCBvcmdhbmljIGNvbXBvdW5kcyBhY2N1bXVsYXRlIGluIGZpc2ggdGlzc3VlcyBhbmQgYWZmZWN0IHRoZWlyIGdyb3d0aCwgcmVwcm9kdWN0aW9uLCBhbmQgb3ZlcmFsbCBoZWFsdGguIFRoaXMgbGVkIHRvIGRlY2xpbmVzIGluIGZpc2ggcG9wdWxhdGlvbnMsIG1ha2luZyBpdCBtb3JlIGRpZmZpY3VsdCBmb3IgZmlzaGVycyB0byBtYWtlIGEgbGl2aW5nLiBJbiBhZGRpdGlvbiB0byBkaXJlY3QgaW1wYWN0cyBvbiBmaXNoIHBvcHVsYXRpb25zLCBwb29yIHdhdGVyIHF1YWxpdHkgYWxzbyBoYXMgaW5kaXJlY3QgZWZmZWN0cyBvbiB0aGUgc29jaWFsIGFuZCBlY29ub21pYyB2dWxuZXJhYmlsaXR5IG9mIFNTRnMuIEZvciBleGFtcGxlLCBjb250YW1pbmF0aW9uIG9mIHdhdGVyIHNvdXJjZXMgbGVkIHRvIHRoZSByZWR1Y2VkIG51bWJlciBvZiBmaXNoIHNwZWNpZXMgcmVkdWNpbmcgdGhlIGFtb3VudCBvZiB0aW1lIGZpc2hlcnMgY2FuIHNwZW5kIG9uIGZpc2hpbmcgYWN0aXZpdGllcy4gVGhpcyBhbHNvIGFmZmVjdGVkIHRoZSBtYXJrZXRhYmlsaXR5IG9mIGZpc2ggcHJvZHVjdHMsIHJlZHVjZWQgaW5jb21lIGFuZCBpbmNyZWFzZWQgcG92ZXJ0eS4gVG8gZnVsbHkgdW5kZXJzdGFuZCB0aGUgaW50ZXJjb25uZWN0aW9ucyBiZXR3ZWVuIHdhdGVyIHF1YWxpdHkgYW5kIHZ1bG5lcmFiaWxpdHkgaW4gU1NGcyBpbiBDaGlsaWthIExhZ29vbiwgaXQgaXMgaW1wb3J0YW50IHRvIGNvbnNpZGVyIGJvdGggZW52aXJvbm1lbnRhbCBhbmQgc29jaWFsIGZhY3RvcnMsIGFzIHdlbGwgYXMgdGhlIGNvbXBsZXggZmVlZGJhY2sgbG9vcHMgYmV0d2VlbiB0aGVzZSBmYWN0b3JzLiBUaGUgc3R1ZHkgaGVscHMgaW4gYnJpZGdpbmcgYSBjcnVjaWFsIGdhcCBpbiBvdXIgdW5kZXJzdGFuZGluZyBvZiB0aGUgcm9sZSBvZiB3YXRlciBxdWFsaXR5IGluIHZ1bG5lcmFiaWxpdHkgYW5hbHlzaXMgd2l0aGluIHJlc291cmNlIGRlcGVuZGVudCBjb21tdW5pdGllcy4gV2UgY29uY2x1ZGUgd2l0aCBrZXkgaW5zaWdodHMgb24gcG9zc2libGUgY29waW5nIHJlc3BvbnNlcyBhbmQgYWRhcHRpdmUgY2FwYWNpdHkgbmVjZXNzYXJ5IGZvciB0aGUgc21hbGwtc2NhbGUgZmlzaGVyaWVzIGNvbW11bml0aWVzIHRvIHRyYW5zaXRpb24gdG93YXJkIHZpYWJpbGl0eS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMzM4OS9mbWFycy4yMDIzLjEwODcyOTYiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vd3d3LmZyb250aWVyc2luLm9yZy9hcnRpY2xlcy8xMC4zMzg5L2ZtYXJzLjIwMjMuMTA4NzI5Ni9wZGYiLCJpc3N1ZWREYXRlIjoiMjAyMy0wNi0yOSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZyb250aWVycyBpbiBNYXJpbmUgU2NpZW5jZSIsInZvbHVtZSI6MTAsImltcGFjdEZhY3RvciI6IjQuMjgifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRnJvbnRpZXJzIE1lZGlhIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo1MSwiY2l0ZWRCeUNvdW50Ijo1fX0sInF1b3RlIjoiV2F0ZXIgcXVhbGl0eSBvZiBDaGlsaWthIExhZ29vbiBoYXMgZGVncmFkZWQgZHVlIHRvIGFudGhyb3BvZ2VuaWMgaW50ZXJ2ZW50aW9ucyBhbmQgbmF0dXJhbCBmb3JjZXMgb2YgY2hhbmdlLiBWYXJpb3VzIGFjdGl2aXRpZXMgaW4gdGhlIENoaWxpa2EgTGFnb29uLCBzdWNoIGFzIGluZHVzdHJpYWwgd2FzdGV3YXRlciBkaXNwb3NhbCwgc2V3YWdlIGR1bXBpbmcsIGFxdWFjdWx0dXJlLCBoeWRyb2xvZ2ljYWwgaW50ZXJ2ZW50aW9ucywgYW5kIGN5Y2xvbmVzLCBhcmUgcHJvZHVjaW5nIHNhbGluaXR5IGNoYW5nZXMsIHNlZGltZW50IGRlcG9zaXRpb24sIG51dHJpZW50IGVucmljaG1lbnQsIGV1dHJvcGhpY2F0aW9uLCBhbmQgZGVhZCB6b25lcywgYXMgc3RhdGVkLiBGaXNoIHN1cnZpdmFsIGlzIGF0IHJpc2sgYXMgYSByZXN1bHQgb2YgdGhlIHNpZ25pZmljYW50IHJlZHVjdGlvbiBpbiBlbnZpcm9ubWVudGFsIGNpcmN1bXN0YW5jZXMsIHdoaWNoIGRpbWluaXNoZXMgZmlzaGVybWVu4oCZcyBpbmNvbWUgYW5kIHB1c2hlcyB0aGVtIG91dCBvZiBzb2NpZXR5LiBUaGVzZSBjYXVzZXMgYWZmZWN0aW5nIHRoZSB3YXRlciBxdWFsaXR5IG9mIHRoZSBMYWdvb24gZGVwaWN0IG1hbnkgZmFjZXMgb2YgdnVsbmVyYWJpbGl0eSBpbiBTU0YgY29tbXVuaXRpZXMuIEFzIHRoZSBDaGlsaWthIExhZ29vbiBlbnZpcm9ubWVudCBpcyBleHBlY3RlZCB0byB1bmRlcmdvIGluY3JlYXNpbmdseSBkcmFzdGljIGNoYW5nZXMgKHN1Y2ggYXMgY2F0Y2ggY29tcG9zaXRpb24sIGNhdGNoIGNhcGFjaXR5LCBiaW9kaXZlcnNpdHkgdmFyaWF0aW9ucywgd2F0ZXIgcXVhbGl0eSwgYW5kIGZpc2hlcnkgcmV2ZW51ZXMpLCBpbnRlcm5hdGlvbmFsIHBvbGljeSBoYXMgYmVjb21lIG1vcmUgaW50ZXJlc3RlZCBpbiBhZGFwdGF0aW9uIG1lYXN1cmVzLiBHaXZlbiBob3cgZGl2ZXJzZSBTU0YgZWNvc3lzdGVtcyBhcmUgZXhwZWN0ZWQgdG8gYmUgYmV0d2VlbiBjb250aW5lbnRzLCB0aGVyZSBpcyBubyDigJxvbmUgc2l6ZSBmaXRzIGFsbOKAnSBhcHByb2FjaC4gVGhlIHBhcGVyIG91dGxpbmVkIHNldmVyYWwgY29waW5nIGFuZCBhZGFwdGF0aW9uIG1lY2hhbmlzbXMgdG8gaW1wcm92ZSB3YXRlciBxdWFsaXR5IGFuZCBjb250aW51ZSBmaXNoaW5nIGluIGEgc3VzdGFpbmFibGUgd2F5LiBBZGFwdGF0aW9uIHN0cmF0ZWdpZXMgY2FuIG9jY2FzaW9uYWxseSBoYXZlIHVuZm9yZXNlZW4gY29uc2VxdWVuY2VzIGZvciBmaXNoZXJpZXMgYW5kIGZpc2hpbmcgY29tbXVuaXRpZXMuIEFueSB1bmV4cGVjdGVkIGNvbnNlcXVlbmNlcyBvZiBhZGFwdGF0aW9uIGNhbiBiZSBtaXRpZ2F0ZWQgdGhyb3VnaCBsb25nLXRlcm0gcGxhbm5pbmcgYW5kIGlkZW50aWZpY2F0aW9uIG9mIGZ1dHVyZSBvcHRpb25zIHRocm91Z2ggc2NlbmFyaW8gYW5hbHlzaXMuIFRoZSBmaW5kaW5ncyBvZiB0aGlzIHBhcGVyIGNhbiBiZSB1dGlsaXplZCB0byBkZXZlbG9wIHBvbGljaWVzIHRvIHByb3RlY3Qgd2F0ZXIgcXVhbGl0eSBhbmQgU1NGcyBpbiBhIHZhcmlldHkgb2YgY29hc3RhbCBsYWdvb24gc2V0dGluZ3MuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjIyLjI4NTAzNjA4NzAzNjEzMywibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ5MzA1MzQzNjI3OTMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjIzLjc3MzUxOTUxNTk5MTIxLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDgzMzY0MTA1MjI0NjEsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjI1LjI1NTU4NDcxNjc5Njg3NSwibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ4OTU1OTE3MzU4Mzk4NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjYuNzQ0MDY4MTQ1NzUxOTUzLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNTA2ODA1NDE5OTIxODc1LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyOC4yMjYxMzMzNDY1NTc2MTcsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40OTA0NjMyNTY4MzU5NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjkuNzE0NjE2Nzc1NTEyNjk1LCJsZWZ0Ijo4LjMzMTU5MjU1OTgxNDQ1Mywid2lkdGgiOjQwLjQ5NDA5ODY2MzMzMDA4LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozMS4xOTY2ODM4ODM2NjY5OTIsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40OTQzOTIzOTUwMTk1MywiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MzIuNjg1MTY1NDA1MjczNDQsImxlZnQiOjguMzMxNTk2Mzc0NTExNzE5LCJ3aWR0aCI6NDAuNDg3NDQ1ODMxMjk4ODMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM0LjE2NzIzMjUxMzQyNzczNCwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40OTc3NDU1MTM5MTYwMTYsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM1LjY1NTcxNTk0MjM4MjgxLCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ5ODYzODE1MzA3NjE3LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozNy4xMzc3ODMwNTA1MzcxMSwibGVmdCI6MTAuNjg1MTg2Mzg2MTA4Mzk4LCJ3aWR0aCI6MzguMTQxMTA1NjUxODU1NDcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM4LjYyNjI2NjQ3OTQ5MjE5LCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4NzE1MjA5OTYwOTM3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDAuMTA4MzMzNTg3NjQ2NDg0LCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4OTg1NjcxOTk3MDcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQxLjU5NjgxMzIwMTkwNDMsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDg4NjY2NTM0NDIzODMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQzLjA3ODg4MDMxMDA1ODU5NCwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40ODE4NjExMTQ1MDE5NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDQuNTY3MzYzNzM5MDEzNjcsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDgzMDU4OTI5NDQzMzYsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQ2LjA0OTQzMDg0NzE2Nzk3LCJsZWZ0Ijo4LjMzMzEwODkwMTk3NzUzOSwid2lkdGgiOjQwLjUwMDc0NzY4MDY2NDA2LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo0Ny41Mzc5MTQyNzYxMjMwNSwibGVmdCI6OC4zMzMxMDg5MDE5Nzc1MzksIndpZHRoIjo0MC40OTU2MDU0Njg3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDkuMDE5OTgxMzg0Mjc3MzQ0LCJsZWZ0Ijo4LjMzMzEwNjA0MDk1NDU5LCJ3aWR0aCI6NDAuNTA2ODE2ODY0MDEzNjcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUwLjUwODQ2NDgxMzIzMjQyLCJsZWZ0Ijo4LjMzMzEwNjA0MDk1NDU5LCJ3aWR0aCI6NDAuNDg4NTA2MzE3MTM4NjcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUxLjk5MDUyODEwNjY4OTQ1LCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDk3NzI2NDQwNDI5NjksImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUzLjQ3OTAxMTUzNTY0NDUzLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6MzMuMTU5NDQ2NzE2MzA4NTk0LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo1NC45NjEwNzg2NDM3OTg4MywibGVmdCI6MTAuNjgzNjcwMDQzOTQ1MzEyLCJ3aWR0aCI6MzguMTM2NTk2Njc5Njg3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NTYuNDQ5NTYyMDcyNzUzOTA2LCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDg3MTQ0NDcwMjE0ODQ0LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEzIn19LHsib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkpvYWNoaW0iLCJmYW1pbHkiOiJKYW5zZW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiR2F2aW4gTC4iLCJmYW1pbHkiOiJTaW1wc29uIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikdlc2EgQS4iLCJmYW1pbHkiOiJXZXloZW5tZXllciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJMYXVyYSBILiIsImZhbWlseSI6IkjDpHJrw7ZuZW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW5kcmV3IE0uIiwiZmFtaWx5IjoiUGF0ZXJzb24ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUGF1bCBBLiBkZWwiLCJmYW1pbHkiOiJHaW9yZ2lvIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ill2ZXMgVC4iLCJmYW1pbHkiOiJQcmFpcmllIn19XSwiaWRzIjp7Imlzc24iOlsiMTc1OC02NzhYIiwiMTc1OC02Nzk4Il0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDM4L3M0MTU1OC0wMjQtMDIwNTgtMyIsIm9wZW5hbGV4IjoiVzQ0MDAyMDgyMDAiLCJleGEiOiJodHRwczovL3d3dy5uYXR1cmUuY29tL2FydGljbGVzL3M0MTU1OC0wMjQtMDIwNTgtMz9lcnJvcj1jb29raWVzX25vdF9zdXBwb3J0ZWQmY29kZT1hNzRhMjVmZi1hNThkLTRkZjgtOTM2Zi0yNjJlMmUzZTkxOTgifSwidGl0bGUiOiJDbGltYXRlLWRyaXZlbiBkZW94eWdlbmF0aW9uIG9mIG5vcnRoZXJuIGxha2VzIiwiYWJzdHJhY3QiOiJBYnN0cmFjdCBPeHlnZW4gZGVwbGV0aW9uIGNvbnN0aXR1dGVzIGEgbWFqb3IgdGhyZWF0IHRvIGxha2UgZWNvc3lzdGVtcyBhbmQgdGhlIHNlcnZpY2VzIHRoZXkgcHJvdmlkZS4gTW9zdCBvZiB0aGUgd29ybGTigJlzIGxha2VzIGFyZSBsb2NhdGVkICZndDs0NcKwIE4sIHdoZXJlIGFjY2VsZXJhdGVkIGNsaW1hdGUgd2FybWluZyBhbmQgZWxldmF0ZWQgY2FyYm9uIGxvYWRzIG1pZ2h0IHNldmVyZWx5IGluY3JlYXNlIHRoZSByaXNrIG9mIGh5cG94aWEsIGJ1dCB0aGlzIGhhcyBub3QgYmVlbiBzeXN0ZW1hdGljYWxseSBleGFtaW5lZC4gSGVyZSBhbmFseXNpcyBvZiAyLjYgbWlsbGlvbiB3YXRlciBxdWFsaXR5IG9ic2VydmF0aW9ucyBmcm9tIDgsMjg4IGxha2VzIHNob3dzIHRoYXQgYmV0d2VlbiAxOTYwIGFuZCAyMDIyLCBtb3N0IG5vcnRoZXJuIGxha2VzIGV4cGVyaWVuY2VkIHJhcGlkIGRlb3h5Z2VuYXRpb24gc3Ryb25nbHkgbGlua2VkIHRvIGNsaW1hdGUtZHJpdmVuIHByb2xvbmdhdGlvbiBvZiBzdW1tZXIgc3RyYXRpZmljYXRpb24uIE94eWdlbiBsZXZlbHMgZGV0ZXJpb3JhdGVkIG1vc3QgaW4gc21hbGwgbGFrZXMgKCZsdDsxMCBoYSkgb3dpbmcgdG8gdGhlaXIgZ3JlYXRlciB2b2x1bWV0cmljIG94eWdlbiBkZW1hbmQgYW5kIHN1cmZhY2Ugd2FybWluZyByYXRlcywgd2hpbGUgdGhlIGxhcmdlc3QgbGFrZXMgZ2FpbmVkIG94eWdlbiB1bmRlciBtaW5pbWFsIHN0cmF0aWZpY2F0aW9uIGNoYW5nZXMgYW5kIGltcHJvdmVkIGFlcmF0aW9uIGF0IHNwcmluZyBvdmVydHVybnMuIFNlYXNvbmFsIG94eWdlbiBjb25zdW1wdGlvbiByYXRlcyBkZWNsaW5lZCwgZGVzcGl0ZSB3aWRlc3ByZWFkIGJyb3duaW5nLiBQcm9saWZlcmF0aW5nIGFub3hpYSBlbmhhbmNlZCBzZWFzb25hbCBpbnRlcm5hbCBsb2FkaW5nIG9mIEMsIFAgYW5kIE4gYnV0IGRlcGxldGVkIFAgbG9uZy10ZXJtLCBpbmRpY2F0aW5nIHRoYXQgZGVveHlnZW5hdGlvbiBjYW4gZXhoYXVzdCByZWRveC1zZW5zaXRpdmUgZnJhY3Rpb25zIG9mIHNlZGltZW50IG51dHJpZW50IHJlc2Vydm9pcnMuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjEwMzgvczQxNTU4LTAyNC0wMjA1OC0zIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAzOC9zNDE1NTgtMDI0LTAyMDU4LTMiLCJpc3N1ZWREYXRlIjoiMjAyNC0wNy0wMSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTEiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6Ik5hdHVyZSBDbGltYXRlIENoYW5nZSIsImlzc3VlIjo4LCJ2b2x1bWUiOjE0LCJpbXBhY3RGYWN0b3IiOiIxNS4wMiJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjgzMiwibGFzdFBhZ2UiOjgzOH19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJOYXR1cmUgUG9ydGZvbGlvIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo4MCwiY2l0ZWRCeUNvdW50IjozN319LCJxdW90ZSI6IkFlcm9iaWMgcmVzcGlyYXRpb24gY2FuIGRlcGxldGUgb3h5Z2VuIGNvbmNlbnRyYXRpb25zIGF0IGRlcHRoIHdoZW4gc2Vhc29uYWwgc3RyYXRpZmljYXRpb24gcHJldmVudHMgYWVyYXRpb24gb2YgdGhlIHdhdGVyIGNvbHVtbls1XShodHRwczovL3d3dy5uYXR1cmUuY29tL2FydGljbGVzL3M0MTU1OC0wMjQtMDIwNTgtMyNyZWYtQ1I1KS4gVGhpcyBkZXRyaW1lbnRhbCBjb25kaXRpb24sIHRlcm1lZCBoeXBveGlhIChET%2BKAiTzigIky4oCJbWfigIls4oiSMSksIGRpbWluaXNoZXMgdGhlIGNvbGQtd2F0ZXIgaGFiaXRhdCBvZiBhcXVhdGljIGxpZmUsIGluY2x1ZGluZyBjb21tZXJjaWFsbHkgaW1wb3J0YW50IGZpc2hbNl0oaHR0cHM6Ly93d3cubmF0dXJlLmNvbS9hcnRpY2xlcy9zNDE1NTgtMDI0LTAyMDU4LTMjcmVmLUNSNikgYW5kIGNhbiBjYXVzZSBtYXNzIG1vcnRhbGl0eSBpZiBwZXJzaXN0ZW50WzddKGh0dHBzOi8vd3d3Lm5hdHVyZS5jb20vYXJ0aWNsZXMvczQxNTU4LTAyNC0wMjA1OC0zI3JlZi1DUjcpLiBBbm94aWEgKERP4oCJPOKAiTAuNeKAiW1n4oCJbOKIkjEpIHByb21vdGVzIHRoZSByZWR1Y3RpdmUgZGlzc29sdXRpb24gb2YgbWluZXJhbC1ib3VuZCBjYXJib24gYW5kIHBob3NwaG9ydXNbOF0oaHR0cHM6Ly93d3cubmF0dXJlLmNvbS9hcnRpY2xlcy9zNDE1NTgtMDI0LTAyMDU4LTMjcmVmLUNSOCksIFs5XShodHRwczovL3d3dy5uYXR1cmUuY29tL2FydGljbGVzL3M0MTU1OC0wMjQtMDIwNTgtMyNyZWYtQ1I5KSB3aGljaCBmdWVscyBoYXJtZnVsIGFsZ2FsIGJsb29tc1sxMF0oaHR0cHM6Ly93d3cubmF0dXJlLmNvbS9hcnRpY2xlcy9zNDE1NTgtMDI0LTAyMDU4LTMjcmVmLUNSMTApIGFuZCBsaW1pdHMgbWljcm9iaWFsIG94aWRhdGlvbiBvZiBtZXRoYW5lLCBhIHBvdGVudCBncmVlbmhvdXNlIGdhc1sxMV0oaHR0cHM6Ly93d3cubmF0dXJlLmNvbS9hcnRpY2xlcy9zNDE1NTgtMDI0LTAyMDU4LTMjcmVmLUNSMTEpLiJ9XQ%3D%3D. Fish yields in Bellandur Lake plummeted by 60-70% post-frothing events, eroding annual revenues estimated at INR 500-1000 crore for peri-urban fisheries (Nair & Nayak, 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hdnlhIFZpa3JhbWFuIiwiZmFtaWx5IjoiTmFpciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQcmF0ZWVwIEt1bWFyIiwiZmFtaWx5IjoiTmF5YWsifX1dLCJpZHMiOnsiaXNzbiI6WyIyMjk2LTc3NDUiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzODkvZm1hcnMuMjAyMy4xMDg3Mjk2Iiwib3BlbmFsZXgiOiJXNDM4MjcyMzEzMyJ9LCJ0aXRsZSI6IlVuY292ZXJpbmcgd2F0ZXIgcXVhbGl0eSBhbmQgZXZhbHVhdGluZyB2dWxuZXJhYmlsaXRpZXMgb2Ygc21hbGwtc2NhbGUgZmlzaGVyaWVzIGluIENoaWxpa2EgTGFnb29uLCBJbmRpYSIsImFic3RyYWN0IjoiU21hbGwtc2NhbGUgZmlzaGVyaWVzIChTU0ZzKSBwbGF5IGEgc3Ryb25nIHJvbGUgaW4gc3VzdGFpbmluZyBtaWxsaW9ucyBvZiBsaXZlbGlob29kcywgZm9vZCBzZWN1cml0eSwgbnV0cml0aW9uLCBhbmQgaW5jb21lIGdsb2JhbGx5IGJ1dCB0aGUgZmlzaGVycyBlbmdhZ2VkIGluIHRoaXMgc2VjdG9yIHNpbXVsdGFuZW91c2x5IGV4cGVyaWVuY2UgaGlnaCBsZXZlbHMgb2YgdnVsbmVyYWJpbGl0eSBhbmQgcHJvY2Vzc2VzIG9mIG1hcmdpbmFsaXNhdGlvbi4gU2V2ZXJhbCBmYWN0b3JzIGFyZSBhdHRyaWJ1dGVkIHRvIHRoZSBtdWx0aWRpbWVuc2lvbmFsIHZ1bG5lcmFiaWxpdGllcyB0aGUgc21hbGwtc2NhbGUgZmlzaGVycyBleHBlcmllbmNlLCBzcGFubmluZyBib3RoIG5hdHVyYWwgKGUuZy4sIG5hdHVyYWwgZGlzYXN0ZXJzLCBlY29zeXN0ZW0gY2hhbmdlKSBhbmQgYW50aHJvcG9nZW5pYyAoZS5nLiwgcG9saWN5IGNoYW5nZSwgaHlkcm9sb2dpY2FsIGludGVydmVudGlvbnMsIGFxdWFjdWx0dXJlKSBwcmVzc3VyZXMuIFdoaWxlIHRoZXJlIGlzIG11Y2ggbGl0ZXJhdHVyZSBvbiB2YXJpb3VzIG5hdHVyYWwgYW5kIGh1bWFuIGRyaXZlcnMgb2YgdnVsbmVyYWJpbGl0eSBpbiBzbWFsbC1zY2FsZSBmaXNoZXJ5IGNvbW11bml0aWVzLCBhbiBhYnNlbmNlIG9mIHJlc2VhcmNoIGNvbm5lY3RpbmcgdnVsbmVyYWJpbGl0eSB3aXRoIHdhdGVyIHF1YWxpdHkgaXMgZXZpZGVudC4gRmlzaGVyIGNvbW11bml0aWVzIG9mdGVuIHRhbGsgYWJvdXQgZmlzaCBpbiByZWxhdGlvbiB0byB0aGUgaGVhbHRoIG9mIHRoZWlyIGFxdWF0aWMgaGFiaXRhdHMgd2hlcmVpbiB3YXRlciBxdWFsaXR5IGlzIHNlZW4gYXMgYSBrZXkgcGFyYW1ldGVyLiBUaGUgbGluayBiZXR3ZWVuIGhlYWx0aHkgZmlzaCBhbmQgZ29vZCBxdWFsaXR5IHdhdGVyIGhhcyBzaWduaWZpY2FudCBpbXBsaWNhdGlvbnMgZm9yIHN0cm9uZyBhbmQgdmlhYmxlIGZpc2hpbmcgY29tbXVuaXRpZXMuIFRoaXMgcGFwZXIgZXhhbWluZXMgdGhlc2UgbGlua3MgZnVydGhlciBieSBmb2N1c2luZyBvbiB0aGUgbmF0dXJlIG9mIHZ1bG5lcmFiaWxpdGllcyBjYXVzZWQgYnkgd2F0ZXIgcXVhbGl0eSBjaGFuZ2VzIGluIHRoZSBzbWFsbC1zY2FsZSBmaXNoZXJ5IHN5c3RlbSBvZiBDaGlsaWthIExhZ29vbiBpbiBJbmRpYS4gV2UgdW5kZXJ0YWtlIGRldGFpbGVkIGFuYWx5c2lzIG9mIHRoZSBpbnZhc2l2ZSBzaHJpbXAgYXF1YWN1bHR1cmUgYWN0aXZpdGllcyBhbmQgaHlkcm9sb2dpY2FsIGludGVydmVudGlvbnMgZm9yIG9wZW5pbmcgb2YgYSBsYWdvb24gaW5sZXQgd2l0aCB0aGUgQmF5IG9mIEJlbmdhbCBhcyB0d28gZG9taW5hbnQgZHJpdmVycyBhZHZlcnNlbHkgaW1wYWN0aW5nIHdhdGVyIHF1YWxpdHkgYW5kIGluY3JlYXNpbmcgdnVsbmVyYWJpbGl0aWVzIG9mIHRoZSBlbnRpcmUgc21hbGwtc2NhbGUgZmlzaGVyaWVzIHNvY2lhbC1lY29sb2dpY2FsIHN5c3RlbS4gT3VyIGFuYWx5c2lzIHN1Z2dlc3RzIHRoYXQgdGhlcmUgYXJlIHN0cm9uZyBpbnRlcmNvbm5lY3Rpb25zIGJldHdlZW4gY2hhbmdlcyBpbiB3YXRlciBxdWFsaXR5IGFuZCB0aGUgbGV2ZWxzIG9mIHZ1bG5lcmFiaWxpdGllcyBpbiB0aGUgU1NGcyBvZiBDaGlsaWthIExhZ29vbi4gUG9sbHV0YW50cyBzdWNoIGFzIHBlc3RpY2lkZXMsIGFuZCBvcmdhbmljIGNvbXBvdW5kcyBhY2N1bXVsYXRlIGluIGZpc2ggdGlzc3VlcyBhbmQgYWZmZWN0IHRoZWlyIGdyb3d0aCwgcmVwcm9kdWN0aW9uLCBhbmQgb3ZlcmFsbCBoZWFsdGguIFRoaXMgbGVkIHRvIGRlY2xpbmVzIGluIGZpc2ggcG9wdWxhdGlvbnMsIG1ha2luZyBpdCBtb3JlIGRpZmZpY3VsdCBmb3IgZmlzaGVycyB0byBtYWtlIGEgbGl2aW5nLiBJbiBhZGRpdGlvbiB0byBkaXJlY3QgaW1wYWN0cyBvbiBmaXNoIHBvcHVsYXRpb25zLCBwb29yIHdhdGVyIHF1YWxpdHkgYWxzbyBoYXMgaW5kaXJlY3QgZWZmZWN0cyBvbiB0aGUgc29jaWFsIGFuZCBlY29ub21pYyB2dWxuZXJhYmlsaXR5IG9mIFNTRnMuIEZvciBleGFtcGxlLCBjb250YW1pbmF0aW9uIG9mIHdhdGVyIHNvdXJjZXMgbGVkIHRvIHRoZSByZWR1Y2VkIG51bWJlciBvZiBmaXNoIHNwZWNpZXMgcmVkdWNpbmcgdGhlIGFtb3VudCBvZiB0aW1lIGZpc2hlcnMgY2FuIHNwZW5kIG9uIGZpc2hpbmcgYWN0aXZpdGllcy4gVGhpcyBhbHNvIGFmZmVjdGVkIHRoZSBtYXJrZXRhYmlsaXR5IG9mIGZpc2ggcHJvZHVjdHMsIHJlZHVjZWQgaW5jb21lIGFuZCBpbmNyZWFzZWQgcG92ZXJ0eS4gVG8gZnVsbHkgdW5kZXJzdGFuZCB0aGUgaW50ZXJjb25uZWN0aW9ucyBiZXR3ZWVuIHdhdGVyIHF1YWxpdHkgYW5kIHZ1bG5lcmFiaWxpdHkgaW4gU1NGcyBpbiBDaGlsaWthIExhZ29vbiwgaXQgaXMgaW1wb3J0YW50IHRvIGNvbnNpZGVyIGJvdGggZW52aXJvbm1lbnRhbCBhbmQgc29jaWFsIGZhY3RvcnMsIGFzIHdlbGwgYXMgdGhlIGNvbXBsZXggZmVlZGJhY2sgbG9vcHMgYmV0d2VlbiB0aGVzZSBmYWN0b3JzLiBUaGUgc3R1ZHkgaGVscHMgaW4gYnJpZGdpbmcgYSBjcnVjaWFsIGdhcCBpbiBvdXIgdW5kZXJzdGFuZGluZyBvZiB0aGUgcm9sZSBvZiB3YXRlciBxdWFsaXR5IGluIHZ1bG5lcmFiaWxpdHkgYW5hbHlzaXMgd2l0aGluIHJlc291cmNlIGRlcGVuZGVudCBjb21tdW5pdGllcy4gV2UgY29uY2x1ZGUgd2l0aCBrZXkgaW5zaWdodHMgb24gcG9zc2libGUgY29waW5nIHJlc3BvbnNlcyBhbmQgYWRhcHRpdmUgY2FwYWNpdHkgbmVjZXNzYXJ5IGZvciB0aGUgc21hbGwtc2NhbGUgZmlzaGVyaWVzIGNvbW11bml0aWVzIHRvIHRyYW5zaXRpb24gdG93YXJkIHZpYWJpbGl0eS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMzM4OS9mbWFycy4yMDIzLjEwODcyOTYiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vd3d3LmZyb250aWVyc2luLm9yZy9hcnRpY2xlcy8xMC4zMzg5L2ZtYXJzLjIwMjMuMTA4NzI5Ni9wZGYiLCJpc3N1ZWREYXRlIjoiMjAyMy0wNi0yOSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZyb250aWVycyBpbiBNYXJpbmUgU2NpZW5jZSIsInZvbHVtZSI6MTAsImltcGFjdEZhY3RvciI6IjQuMjgifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRnJvbnRpZXJzIE1lZGlhIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo1MSwiY2l0ZWRCeUNvdW50Ijo1fX0sInF1b3RlIjoiV2F0ZXIgcXVhbGl0eSBvZiBDaGlsaWthIExhZ29vbiBoYXMgZGVncmFkZWQgZHVlIHRvIGFudGhyb3BvZ2VuaWMgaW50ZXJ2ZW50aW9ucyBhbmQgbmF0dXJhbCBmb3JjZXMgb2YgY2hhbmdlLiBWYXJpb3VzIGFjdGl2aXRpZXMgaW4gdGhlIENoaWxpa2EgTGFnb29uLCBzdWNoIGFzIGluZHVzdHJpYWwgd2FzdGV3YXRlciBkaXNwb3NhbCwgc2V3YWdlIGR1bXBpbmcsIGFxdWFjdWx0dXJlLCBoeWRyb2xvZ2ljYWwgaW50ZXJ2ZW50aW9ucywgYW5kIGN5Y2xvbmVzLCBhcmUgcHJvZHVjaW5nIHNhbGluaXR5IGNoYW5nZXMsIHNlZGltZW50IGRlcG9zaXRpb24sIG51dHJpZW50IGVucmljaG1lbnQsIGV1dHJvcGhpY2F0aW9uLCBhbmQgZGVhZCB6b25lcywgYXMgc3RhdGVkLiBGaXNoIHN1cnZpdmFsIGlzIGF0IHJpc2sgYXMgYSByZXN1bHQgb2YgdGhlIHNpZ25pZmljYW50IHJlZHVjdGlvbiBpbiBlbnZpcm9ubWVudGFsIGNpcmN1bXN0YW5jZXMsIHdoaWNoIGRpbWluaXNoZXMgZmlzaGVybWVu4oCZcyBpbmNvbWUgYW5kIHB1c2hlcyB0aGVtIG91dCBvZiBzb2NpZXR5LiBUaGVzZSBjYXVzZXMgYWZmZWN0aW5nIHRoZSB3YXRlciBxdWFsaXR5IG9mIHRoZSBMYWdvb24gZGVwaWN0IG1hbnkgZmFjZXMgb2YgdnVsbmVyYWJpbGl0eSBpbiBTU0YgY29tbXVuaXRpZXMuIEFzIHRoZSBDaGlsaWthIExhZ29vbiBlbnZpcm9ubWVudCBpcyBleHBlY3RlZCB0byB1bmRlcmdvIGluY3JlYXNpbmdseSBkcmFzdGljIGNoYW5nZXMgKHN1Y2ggYXMgY2F0Y2ggY29tcG9zaXRpb24sIGNhdGNoIGNhcGFjaXR5LCBiaW9kaXZlcnNpdHkgdmFyaWF0aW9ucywgd2F0ZXIgcXVhbGl0eSwgYW5kIGZpc2hlcnkgcmV2ZW51ZXMpLCBpbnRlcm5hdGlvbmFsIHBvbGljeSBoYXMgYmVjb21lIG1vcmUgaW50ZXJlc3RlZCBpbiBhZGFwdGF0aW9uIG1lYXN1cmVzLiBHaXZlbiBob3cgZGl2ZXJzZSBTU0YgZWNvc3lzdGVtcyBhcmUgZXhwZWN0ZWQgdG8gYmUgYmV0d2VlbiBjb250aW5lbnRzLCB0aGVyZSBpcyBubyDigJxvbmUgc2l6ZSBmaXRzIGFsbOKAnSBhcHByb2FjaC4gVGhlIHBhcGVyIG91dGxpbmVkIHNldmVyYWwgY29waW5nIGFuZCBhZGFwdGF0aW9uIG1lY2hhbmlzbXMgdG8gaW1wcm92ZSB3YXRlciBxdWFsaXR5IGFuZCBjb250aW51ZSBmaXNoaW5nIGluIGEgc3VzdGFpbmFibGUgd2F5LiBBZGFwdGF0aW9uIHN0cmF0ZWdpZXMgY2FuIG9jY2FzaW9uYWxseSBoYXZlIHVuZm9yZXNlZW4gY29uc2VxdWVuY2VzIGZvciBmaXNoZXJpZXMgYW5kIGZpc2hpbmcgY29tbXVuaXRpZXMuIEFueSB1bmV4cGVjdGVkIGNvbnNlcXVlbmNlcyBvZiBhZGFwdGF0aW9uIGNhbiBiZSBtaXRpZ2F0ZWQgdGhyb3VnaCBsb25nLXRlcm0gcGxhbm5pbmcgYW5kIGlkZW50aWZpY2F0aW9uIG9mIGZ1dHVyZSBvcHRpb25zIHRocm91Z2ggc2NlbmFyaW8gYW5hbHlzaXMuIFRoZSBmaW5kaW5ncyBvZiB0aGlzIHBhcGVyIGNhbiBiZSB1dGlsaXplZCB0byBkZXZlbG9wIHBvbGljaWVzIHRvIHByb3RlY3Qgd2F0ZXIgcXVhbGl0eSBhbmQgU1NGcyBpbiBhIHZhcmlldHkgb2YgY29hc3RhbCBsYWdvb24gc2V0dGluZ3MuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjIyLjI4NTAzNjA4NzAzNjEzMywibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ5MzA1MzQzNjI3OTMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjIzLjc3MzUxOTUxNTk5MTIxLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDgzMzY0MTA1MjI0NjEsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjI1LjI1NTU4NDcxNjc5Njg3NSwibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ4OTU1OTE3MzU4Mzk4NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjYuNzQ0MDY4MTQ1NzUxOTUzLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNTA2ODA1NDE5OTIxODc1LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyOC4yMjYxMzMzNDY1NTc2MTcsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40OTA0NjMyNTY4MzU5NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjkuNzE0NjE2Nzc1NTEyNjk1LCJsZWZ0Ijo4LjMzMTU5MjU1OTgxNDQ1Mywid2lkdGgiOjQwLjQ5NDA5ODY2MzMzMDA4LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozMS4xOTY2ODM4ODM2NjY5OTIsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40OTQzOTIzOTUwMTk1MywiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MzIuNjg1MTY1NDA1MjczNDQsImxlZnQiOjguMzMxNTk2Mzc0NTExNzE5LCJ3aWR0aCI6NDAuNDg3NDQ1ODMxMjk4ODMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM0LjE2NzIzMjUxMzQyNzczNCwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40OTc3NDU1MTM5MTYwMTYsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM1LjY1NTcxNTk0MjM4MjgxLCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ5ODYzODE1MzA3NjE3LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozNy4xMzc3ODMwNTA1MzcxMSwibGVmdCI6MTAuNjg1MTg2Mzg2MTA4Mzk4LCJ3aWR0aCI6MzguMTQxMTA1NjUxODU1NDcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM4LjYyNjI2NjQ3OTQ5MjE5LCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4NzE1MjA5OTYwOTM3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDAuMTA4MzMzNTg3NjQ2NDg0LCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4OTg1NjcxOTk3MDcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQxLjU5NjgxMzIwMTkwNDMsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDg4NjY2NTM0NDIzODMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQzLjA3ODg4MDMxMDA1ODU5NCwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40ODE4NjExMTQ1MDE5NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDQuNTY3MzYzNzM5MDEzNjcsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDgzMDU4OTI5NDQzMzYsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQ2LjA0OTQzMDg0NzE2Nzk3LCJsZWZ0Ijo4LjMzMzEwODkwMTk3NzUzOSwid2lkdGgiOjQwLjUwMDc0NzY4MDY2NDA2LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo0Ny41Mzc5MTQyNzYxMjMwNSwibGVmdCI6OC4zMzMxMDg5MDE5Nzc1MzksIndpZHRoIjo0MC40OTU2MDU0Njg3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDkuMDE5OTgxMzg0Mjc3MzQ0LCJsZWZ0Ijo4LjMzMzEwNjA0MDk1NDU5LCJ3aWR0aCI6NDAuNTA2ODE2ODY0MDEzNjcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUwLjUwODQ2NDgxMzIzMjQyLCJsZWZ0Ijo4LjMzMzEwNjA0MDk1NDU5LCJ3aWR0aCI6NDAuNDg4NTA2MzE3MTM4NjcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUxLjk5MDUyODEwNjY4OTQ1LCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDk3NzI2NDQwNDI5NjksImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUzLjQ3OTAxMTUzNTY0NDUzLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6MzMuMTU5NDQ2NzE2MzA4NTk0LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo1NC45NjEwNzg2NDM3OTg4MywibGVmdCI6MTAuNjgzNjcwMDQzOTQ1MzEyLCJ3aWR0aCI6MzguMTM2NTk2Njc5Njg3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NTYuNDQ5NTYyMDcyNzUzOTA2LCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDg3MTQ0NDcwMjE0ODQ0LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEzIn19XQ%3D%3D. Planktivorous species briefly surge under low-DO regimes, but long-term anoxia favors invasives, collapsing biodiversity and commercial catches of native carp and catfishes (Jilbert et al., 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRvbSIsImZhbWlseSI6IkppbGJlcnQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUmFvdWzigJBNYXJpZSIsImZhbWlseSI6IkNvdXR1cmUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQnJpYW4iLCJmYW1pbHkiOiJIdXNlciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLYWxldmkiLCJmYW1pbHkiOiJTYWxvbmVuIn19XSwiaWRzIjp7Imlzc24iOlsiMDAxOC04MTU4IiwiMTU3My01MTE3Il0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDA3L3MxMDc1MC0wMjAtMDQ0NTcteCIsIm1hZyI6IjMxMDA0NTkwNTAiLCJvcGVuYWxleCI6IlczMTAwNDU5MDUwIn0sInRpdGxlIjoiUHJlZmFjZTogUmVzdG9yYXRpb24gb2YgZXV0cm9waGljIGxha2VzOiBjdXJyZW50IHByYWN0aWNlcyBhbmQgZnV0dXJlIGNoYWxsZW5nZXMiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAwNy9zMTA3NTAtMDIwLTA0NDU3LXgiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vbGluay5zcHJpbmdlci5jb20vY29udGVudC9wZGYvMTAuMTAwNy9zMTA3NTAtMDIwLTA0NDU3LXgucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjAtMTEtMTYiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA3IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJIeWRyb2Jpb2xvZ2lhIiwiaXNzdWUiOjIxLCJ2b2x1bWUiOjg0NywiaW1wYWN0RmFjdG9yIjoiMy4wMCJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjQzNDMsImxhc3RQYWdlIjo0MzU3fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlNwcmluZ2VyIFNjaWVuY2UrQnVzaW5lc3MgTWVkaWEifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjk1LCJjaXRlZEJ5Q291bnQiOjU4fX0sInF1b3RlIjoiU2ltaWxhciBkZWdyYWRhdGlvbiBvZiBmcmVzaHdhdGVyIGVjb3N5c3RlbXMgaXMgb2JzZXJ2ZWQgd29ybGR3aWRlLCB3aXRoIHRoZSB0aW1lbGluZSBvZiBodW1hbiBwcmVzc3VyZSBkaWN0YXRlZCBieSB0cmVuZHMgaW4gdXJiYW5pemF0aW9uIGFuZCBpbnRlbnNpdmUgYWdyaWN1bHR1cmUgaW4gZGlmZmVyZW50IGdlb2dyYXBoaWMgcmVnaW9ucyAoZS5nLiwgVmVubmVtbyBldCBhbC4sIDIwMDkpLiBUaGUgcHJpbmNpcGFsIGRyaXZlciBvZiBlY29zeXN0ZW0gc3RyZXNzIGluIG1vc3QgYXF1YXRpYyBzeXN0ZW1zIGlzIGV1dHJvcGhpY2F0aW9uLCB0aGUgZXhjZXNzIGdyb3d0aCBvZiBhbGdhZSBkcml2ZW4gYnkgYW50aHJvcG9nZW5pYyBpbnB1dHMgb2YgdGhlIG1hY3JvbnV0cmllbnRzIG5pdHJvZ2VuIChOKSBhbmQgcGhvc3Bob3J1cyAoUCkgKFNtaXRoLCAyMDAzOyBCaXJrIGV0IGFsLiwgMjAyMCkuIE51dHJpZW50cyBhcmUgZW1pdHRlZCBmcm9tIGJvdGggcG9pbnQgYW5kIGRpZmZ1c2Ugc291cmNlcyBpbiB0aGUgaHVtYW4gZW52aXJvbm1lbnQgYW5kIGVudGVyIGFxdWF0aWMgc3lzdGVtcyB2aWEgc3VyZmFjZSBydW5vZmYsIGFzIHdlbGwgYXMgZ3JvdW5kLXdhdGVyIGFuZCBhdG1vc3BoZXJpYyBkZXBvc2l0aW9uIChQYWVybCwgMTk5NykuIEV1dHJvcGhpY2F0aW9uIG9mdGVuIGxlYWRzIHRvIHRoZSBkb21pbmFuY2Ugb2YgcHJpbWFyeSBwcm9kdWN0aW9uIGJ5IGN5YW5vYmFjdGVyaWEsIGluY2x1ZGluZyB0b3hpYyBnZW5lcmEgc3VjaCBhcyBEb2xpY2hvc3Blcm11bSwgQXBoYW5pem9tZW5vbiwgYW5kIE1pY3JvY3lzdGlzIChIdWlzbWFuIGV0IGFsLiwgMjAxOCksIGFzIHdlbGwgYXMgb3h5Z2VuIGRlcGxldGlvbiBhbmQgaGFiaXRhdCBkZWdyYWRhdGlvbiBpbiBsYWtlcyAoQXJlbmQgZXQgYWwuLCAyMDExKS4gQmVjYXVzZSBsYWtlcyBwcm92aWRlIGEgcmFuZ2Ugb2YgY3J1Y2lhbCBlY29zeXN0ZW0gc2VydmljZXMgKFdpbHNvbiAmIENhcnBlbnRlciwgMTk5OSksIHRoZXJlIGlzIGEgZ3Jvd2luZyBuZWVkIGZvciBtYW5hZ2VtZW50IGFwcHJvYWNoZXMgdGhhdCByZXN0b3JlIHdhdGVyIHF1YWxpdHkgaW4gZnJlc2h3YXRlciBzeXN0ZW1zLiBXaXRoaW4gdGhlIGZyYW1ld29yayBvZiBVbml0ZWQgTmF0aW9ucyBTdXN0YWluYWJsZSBEZXZlbG9wLW1lbnQgR29hbCA2IChDbGVhbiBXYXRlciBhbmQgU2FuaXRhdGlvbiksIHNldmVyYWwgZGlzdGluY3QgdGFyZ2V0cyByZWxhdGUgdG8gdGhlIGltcHJvdmVtZW50IG9mIHdhdGVyIHF1YWxpdHkgYW5kIHByb3RlY3Rpb24gb2YgYXF1YXRpYyBlY29zeXN0ZW1zIGJ5IDIwMzAgKFVuaXRlZCBOYXRpb25zLCAyMDE1KS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyOC4yOTg4NjQzNjQ2MjQwMjMsImxlZnQiOjguNTQ5MjA1NzgwMDI5Mjk3LCJ3aWR0aCI6MzkuMzcxNDk0MjkzMjEyODksImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MjkuOTkxMTM0NjQzNTU0Njg4LCJsZWZ0Ijo4LjU0OTIwNTc4MDAyOTI5Nywid2lkdGgiOjM5LjM2NTExOTkzNDA4MjAzLCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMxLjY4MzQwNjgyOTgzMzk4NCwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNjk2NjMyMzg1MjUzOSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozMy4zNjc5NzMzMjc2MzY3MiwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNzU2NzkwMTYxMTMyOCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNS4wNjAyNDkzMjg2MTMyOCwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNzA5NjQwNTAyOTI5NywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNi43NTI1MTc3MDAxOTUzMSwibGVmdCI6OC41NDkyMDg2NDEwNTIyNDYsIndpZHRoIjozOS4zNzY5NjA3NTQzOTQ1MywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC40MzcwODgwMTI2OTUzMSwibGVmdCI6OC41NDkyMDg2NDEwNTIyNDYsIndpZHRoIjozOS4zNjY1Njk1MTkwNDI5NywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0MC4xMjkzNTYzODQyNzczNDQsImxlZnQiOjguNTQ5MjA4NjQxMDUyMjQ2LCJ3aWR0aCI6MzkuMzY0Mzc5ODgyODEyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0MS44MjE2MjQ3NTU4NTkzNzUsImxlZnQiOjguNTQ5MjA4NjQxMDUyMjQ2LCJ3aWR0aCI6MzkuMzY1NjczMDY1MTg1NTUsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDMuNTA2MTk1MDY4MzU5Mzc1LCJsZWZ0Ijo4LjU0OTM4OTgzOTE3MjM2Mywid2lkdGgiOjM5LjM3MDM5MTg0NTcwMzEyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0NS4xOTg0NjM0Mzk5NDE0MDYsImxlZnQiOjguNTQ5NTcyOTQ0NjQxMTEzLCJ3aWR0aCI6MzkuMzY1ODQ0NzI2NTYyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ni44OTA3MzU2MjYyMjA3LCJsZWZ0Ijo4LjU0OTU3Mjk0NDY0MTExMywid2lkdGgiOjM5LjM3Mjk1NTMyMjI2NTYyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0OC41NzUzMDIxMjQwMjM0NCwibGVmdCI6OC41NDk1NzI5NDQ2NDExMTMsIndpZHRoIjozOS4zNzI0MjEyNjQ2NDg0NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1MC4yNjc1NzgxMjUsImxlZnQiOjguNTQ5NTc2NzU5MzM4Mzc5LCJ3aWR0aCI6MzkuMzcxMTI0MjY3NTc4MTI1LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjUxLjk1OTg0MjY4MTg4NDc2NiwibGVmdCI6OC41NDk1NzY3NTkzMzgzNzksIndpZHRoIjozOS4zNjc4MzU5OTg1MzUxNTYsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTMuNTc0MTE5NTY3ODcxMDk0LCJsZWZ0Ijo4LjU0OTU3Njc1OTMzODM3OSwid2lkdGgiOjM5LjM3MDc3NzEzMDEyNjk1LCJoZWlnaHQiOjEuMzM2ODk1NTg1MDYwMTE5Nn19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjU1LjI2NjM5MTc1NDE1MDM5LCJsZWZ0Ijo4LjU0OTc2MDgxODQ4MTQ0NSwid2lkdGgiOjM5LjM2NjU4MDk2MzEzNDc2NiwiaGVpZ2h0IjoxLjMzNjg5NTU4NTA2MDExOTZ9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1Ny4wMjg5NTM1NTIyNDYwOTQsImxlZnQiOjguNTQ5OTQ0ODc3NjI0NTEyLCJ3aWR0aCI6MzkuMzYyMDI2MjE0NTk5NjEsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTguNzEzNTIzODY0NzQ2MDk0LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjkuMjk5OTgyMDcwOTIyODUyLCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjYwLjQwNTc5MjIzNjMyODEyNSwibGVmdCI6MTAuNjIyNDU1NTk2OTIzODI4LCJ3aWR0aCI6MzcuMjg3ODY4NDk5NzU1ODYsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NjIuMDk4MDYwNjA3OTEwMTU2LCJsZWZ0Ijo4LjU0OTk0MzkyMzk1MDE5NSwid2lkdGgiOjM5LjM3MzMyNTM0NzkwMDM5LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjYzLjc4MjYzMDkyMDQxMDE1NiwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzMxMjY5ODM2NDI1OCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2NS40NzQ5MDY5MjEzODY3MiwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNjUxMTk5MzQwODIwMywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2Ny4xNjcxNzUyOTI5Njg3NSwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNjk0ODAxMzMwNTY2NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2OC43ODE0NDgzNjQyNTc4MSwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzczMzg0MDk0MjM4MywiaGVpZ2h0IjoxLjMzNjg5MTUzMTk0NDI3NX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjcwLjU0NDAxMzk3NzA1MDc4LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjM5LjM2OTMwODQ3MTY3OTY5LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjcyLjIzNjI4OTk3ODAyNzM0LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjM5LjM3MjAzMjE2NTUyNzM0NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo3My45MjA4NDUwMzE3MzgyOCwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzEzMDczNzMwNDY4NzUsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNDM0NCJ9fV0%3D. Fishers, often artisanal operators with marginal investments, face cascading losses: unsold quotas, damaged gear from scum entanglement, and market devaluation due to toxin fears (Kumar & Mahajan, 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlBhd2FuIiwiZmFtaWx5IjoiS3VtYXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW1icmlzaCBLdW1hciIsImZhbWlseSI6Ik1haGFqYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyNTIzLTM5NjMiLCIyNTIzLTM5NzEiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMDcvczQyNDUyLTAyMC0zMDgyLTgiLCJtYWciOiIzMDM1OTYwMjI4Iiwib3BlbmFsZXgiOiJXMzAzNTk2MDIyOCJ9LCJ0aXRsZSI6IlRyb3BoaWMgc3RhdHVzIGFuZCBpdHMgcmVndWxhdGluZyBmYWN0b3IgZGV0ZXJtaW5hdGlvbiBhdCB0aGUgUmV3YWxzYXIgTGFrZSwgbm9ydGh3ZXN0IEhpbWFsYXlhIChIUCksIEluZGlhLCBiYXNlZCBvbiBzZWxlY3RlZCBwYXJhbWV0ZXJzIGFuZCBtdWx0aXZhcmlhdGUgc3RhdGlzdGljYWwgYW5hbHlzaXMiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAwNy9zNDI0NTItMDIwLTMwODItOCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9saW5rLnNwcmluZ2VyLmNvbS9jb250ZW50L3BkZi8xMC4xMDA3L3M0MjQ1Mi0wMjAtMzA4Mi04LnBkZiIsImlzc3VlZERhdGUiOiIyMDIwLTA2LTIyIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiU04gQXBwbGllZCBTY2llbmNlcyIsImlzc3VlIjo3LCJ2b2x1bWUiOjIsImltcGFjdEZhY3RvciI6IjcuMzUifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiU3ByaW5nZXIgTmF0dXJlIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo0MCwiY2l0ZWRCeUNvdW50IjoxOX19LCJxdW90ZSI6IkNoYW5naW5nIHF1YWxpdHkgaGFzIGNyZWF0ZWQgdGhlIHdhdGVyIHNjYXJjaXR5IGFuZCAgYSBodWdlIG1hc3Mgb2YgcG9wdWxhdGlvbiB0aHVzIGJlZW4gZGVwcml2ZWQgZnJvbSB1c2luZyAgdGhlIGZyZXNod2F0ZXIgZm9yIHZhcmlvdXMgcHVycG9zZXMgWzMsIDRdLiBIb3dldmVyLCB0aGUgIG9ibm94aW91cyBtYXRlcmlhbHMgdGhhdCBzdGF5IGluIHdhdGVyIGJvZGllcyBmb3IgYSBsb25nLXRpbWUgcGVyaW9kIGJyaW5nIGNoYW5nZXMgaW4gdGhlIHBoeXNpb2NoZW1pY2FsIGNoYXItYWN0ZXJpc3RpY3MgYXMgd2VsbCBhcyB0aGUgbmF0dXJhbCBzY2VuYXJpbyBvZiB0aGUgbGFrZXMgWzVdLiAgVGhlIEhpbWFsYXlhbiBsYWtlcyBhcmUgdW5kZXIgc3RyZXNzIGR1ZSB0byBtaXNtYW5hZ2VkIGh1bWFuIGFjdGl2aXRpZXMsIHdoaWNoIGluY3JlYXNlIG51dHJpZW50cyBhbmQgcG9sbHV0aW9uIGluIHRlcm1zIG9mIGhlYXZ5IG1ldGFscyB0b3hpY2l0eSBhbmQgYmFjdGVyaWFsIGNvbnRhbWluYXRpb24gIFs24oCTOF0uIFRoZSBtYW5hZ2VtZW50IGFuZCBwcmVzZXJ2YXRpb24gb2YgdGhlIGxha2VzIGF0ICBwcmVzZW50IGhhdmUgYmVjb21lIGEgZ3JlYXQgY2hhbGxlbmdlIGZvciBlbnZpcm9ubWVudGFsLWlzdHMgYW5kIG1hbmFnZXJzIFs1LCA5XS4gVGhlIGV1dHJvcGhpY2F0aW9uIGlzIGFsc28gYSBtYWluIHByb2JsZW0gb2YgdGhlICBmcmVzaHdhdGVyIGxha2VzLCBhbmQgaXQgaXMgc3BlY2lmaWVkIGZvciB0aGUgbGFrZXMgaGF2aW5nICBudXRyaWVudCBlbnJpY2htZW50LiBJdCBjYXVzZXMgZXhjZXNzaXZlIGdyb3d0aCBvZiBhbGdhZSAgYW5kIGlzIGNvbnNpZGVyZWQgdG8gYmUgcmVzcG9uc2libGUgZm9yIGRlZ3JhZGluZyB0aGUgIHF1YWxpdGF0aXZlIG5hdHVyZSBvZiB0aGUgbGFrZXMgWzUsIDEwXS4gVGhlIGV1dHJvcGhpY2F0aW9uICBpcyBmb2xsb3dlZCBieSB0aGUgYWxnYWwgYmxvb20sIHdoaWNoIGdlbmVyYXRlcyB0b3hpbiwgIHdvcnNlbnMgd2F0ZXIgcXVhbGl0eSwgZGVwbGV0ZXMgZGlzc29sdmVkIG94eWdlbiAoRE8pICogUGF3YW4gS3VtYXIsIHBhd2FuLmV2c0BnbWFpbC5jb20gfCAxRGVwYXJ0bWVudCBvZiBFbnZpcm9ubWVudGFsIFNjaWVuY2VzLCBDZW50cmFsIFVuaXZlcnNpdHkgb2YgSGltYWNoYWwgUHJhZGVzaCwgRGhhcmFtc2hhbGEgMTc2MjE1LCBJbmRpYS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2OC41Nzg0NjA2OTMzNTkzOCwibGVmdCI6OC41NzE0MTg3NjIyMDcwMzEsIndpZHRoIjo0MC4zMjc1MTQ2NDg0Mzc1LCJoZWlnaHQiOjEuNTE5ODU1MjYwODQ4OTk5fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NzAuMTU4OTk2NTgyMDMxMjUsImxlZnQiOjguNTY5NzM4Mzg4MDYxNTIzLCJ3aWR0aCI6NDAuMzU0NzI0ODg0MDMzMiwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjcxLjczOTU0NzcyOTQ5MjE5LCJsZWZ0Ijo4LjU2OTczODM4ODA2MTUyMywid2lkdGgiOjQwLjM0OTY5MzI5ODMzOTg0NCwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjczLjMyMDA5MTI0NzU1ODYsImxlZnQiOjguNTY4MDU5OTIxMjY0NjQ4LCJ3aWR0aCI6NDAuMDIwMDg4MTk1ODAwNzgsImhlaWdodCI6MS41MTk4NTUyNjA4NDg5OTl9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo3NC45MDA2NDIzOTUwMTk1MywibGVmdCI6OC41NjgwNTk5MjEyNjQ2NDgsIndpZHRoIjozOS45OTk5Mjc1MjA3NTE5NSwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjc2LjQ4MTE4NTkxMzA4NTk0LCJsZWZ0Ijo4LjU2ODA1NzA2MDI0MTcsIndpZHRoIjo0MC4zNTcwNzQ3Mzc1NDg4MywiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjc4LjA2MTcyOTQzMTE1MjM0LCJsZWZ0Ijo4LjU2OTczNjQ4MDcxMjg5LCJ3aWR0aCI6NDAuMzUzNzEzOTg5MjU3ODEsImhlaWdodCI6MS41MTk4NTUyNjA4NDg5OTl9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2MC42NzU3Mjc4NDQyMzgyOCwibGVmdCI6NTEuNDI3MTY1OTg1MTA3NDIsIndpZHRoIjo0MC4zNzg1NjY3NDE5NDMzNiwiaGVpZ2h0IjoxOC45MDU4NTUxNzg4MzMwMDh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo4MS45MjczMzc2NDY0ODQzOCwibGVmdCI6OC40MDE0OTc4NDA4ODEzNDgsIndpZHRoIjo3My45NTQ2OTY2NTUyNzM0NCwiaGVpZ2h0IjoyLjY3ODc2MjY3NDMzMTY2NX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEifX1d. In Rewalsar Lake, Microcystis blooms linked to cattle deaths extended to fishery bans, amplifying income deficits by 40-50% in affected seasons (Kumar & Mahajan, 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlBhd2FuIiwiZmFtaWx5IjoiS3VtYXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW1icmlzaCBLdW1hciIsImZhbWlseSI6Ik1haGFqYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyNTIzLTM5NjMiLCIyNTIzLTM5NzEiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMDcvczQyNDUyLTAyMC0zMDgyLTgiLCJtYWciOiIzMDM1OTYwMjI4Iiwib3BlbmFsZXgiOiJXMzAzNTk2MDIyOCJ9LCJ0aXRsZSI6IlRyb3BoaWMgc3RhdHVzIGFuZCBpdHMgcmVndWxhdGluZyBmYWN0b3IgZGV0ZXJtaW5hdGlvbiBhdCB0aGUgUmV3YWxzYXIgTGFrZSwgbm9ydGh3ZXN0IEhpbWFsYXlhIChIUCksIEluZGlhLCBiYXNlZCBvbiBzZWxlY3RlZCBwYXJhbWV0ZXJzIGFuZCBtdWx0aXZhcmlhdGUgc3RhdGlzdGljYWwgYW5hbHlzaXMiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAwNy9zNDI0NTItMDIwLTMwODItOCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9saW5rLnNwcmluZ2VyLmNvbS9jb250ZW50L3BkZi8xMC4xMDA3L3M0MjQ1Mi0wMjAtMzA4Mi04LnBkZiIsImlzc3VlZERhdGUiOiIyMDIwLTA2LTIyIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiU04gQXBwbGllZCBTY2llbmNlcyIsImlzc3VlIjo3LCJ2b2x1bWUiOjIsImltcGFjdEZhY3RvciI6IjcuMzUifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiU3ByaW5nZXIgTmF0dXJlIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo0MCwiY2l0ZWRCeUNvdW50IjoxOX19LCJxdW90ZSI6IkNoYW5naW5nIHF1YWxpdHkgaGFzIGNyZWF0ZWQgdGhlIHdhdGVyIHNjYXJjaXR5IGFuZCAgYSBodWdlIG1hc3Mgb2YgcG9wdWxhdGlvbiB0aHVzIGJlZW4gZGVwcml2ZWQgZnJvbSB1c2luZyAgdGhlIGZyZXNod2F0ZXIgZm9yIHZhcmlvdXMgcHVycG9zZXMgWzMsIDRdLiBIb3dldmVyLCB0aGUgIG9ibm94aW91cyBtYXRlcmlhbHMgdGhhdCBzdGF5IGluIHdhdGVyIGJvZGllcyBmb3IgYSBsb25nLXRpbWUgcGVyaW9kIGJyaW5nIGNoYW5nZXMgaW4gdGhlIHBoeXNpb2NoZW1pY2FsIGNoYXItYWN0ZXJpc3RpY3MgYXMgd2VsbCBhcyB0aGUgbmF0dXJhbCBzY2VuYXJpbyBvZiB0aGUgbGFrZXMgWzVdLiAgVGhlIEhpbWFsYXlhbiBsYWtlcyBhcmUgdW5kZXIgc3RyZXNzIGR1ZSB0byBtaXNtYW5hZ2VkIGh1bWFuIGFjdGl2aXRpZXMsIHdoaWNoIGluY3JlYXNlIG51dHJpZW50cyBhbmQgcG9sbHV0aW9uIGluIHRlcm1zIG9mIGhlYXZ5IG1ldGFscyB0b3hpY2l0eSBhbmQgYmFjdGVyaWFsIGNvbnRhbWluYXRpb24gIFs24oCTOF0uIFRoZSBtYW5hZ2VtZW50IGFuZCBwcmVzZXJ2YXRpb24gb2YgdGhlIGxha2VzIGF0ICBwcmVzZW50IGhhdmUgYmVjb21lIGEgZ3JlYXQgY2hhbGxlbmdlIGZvciBlbnZpcm9ubWVudGFsLWlzdHMgYW5kIG1hbmFnZXJzIFs1LCA5XS4gVGhlIGV1dHJvcGhpY2F0aW9uIGlzIGFsc28gYSBtYWluIHByb2JsZW0gb2YgdGhlICBmcmVzaHdhdGVyIGxha2VzLCBhbmQgaXQgaXMgc3BlY2lmaWVkIGZvciB0aGUgbGFrZXMgaGF2aW5nICBudXRyaWVudCBlbnJpY2htZW50LiBJdCBjYXVzZXMgZXhjZXNzaXZlIGdyb3d0aCBvZiBhbGdhZSAgYW5kIGlzIGNvbnNpZGVyZWQgdG8gYmUgcmVzcG9uc2libGUgZm9yIGRlZ3JhZGluZyB0aGUgIHF1YWxpdGF0aXZlIG5hdHVyZSBvZiB0aGUgbGFrZXMgWzUsIDEwXS4gVGhlIGV1dHJvcGhpY2F0aW9uICBpcyBmb2xsb3dlZCBieSB0aGUgYWxnYWwgYmxvb20sIHdoaWNoIGdlbmVyYXRlcyB0b3hpbiwgIHdvcnNlbnMgd2F0ZXIgcXVhbGl0eSwgZGVwbGV0ZXMgZGlzc29sdmVkIG94eWdlbiAoRE8pICogUGF3YW4gS3VtYXIsIHBhd2FuLmV2c0BnbWFpbC5jb20gfCAxRGVwYXJ0bWVudCBvZiBFbnZpcm9ubWVudGFsIFNjaWVuY2VzLCBDZW50cmFsIFVuaXZlcnNpdHkgb2YgSGltYWNoYWwgUHJhZGVzaCwgRGhhcmFtc2hhbGEgMTc2MjE1LCBJbmRpYS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2OC41Nzg0NjA2OTMzNTkzOCwibGVmdCI6OC41NzE0MTg3NjIyMDcwMzEsIndpZHRoIjo0MC4zMjc1MTQ2NDg0Mzc1LCJoZWlnaHQiOjEuNTE5ODU1MjYwODQ4OTk5fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NzAuMTU4OTk2NTgyMDMxMjUsImxlZnQiOjguNTY5NzM4Mzg4MDYxNTIzLCJ3aWR0aCI6NDAuMzU0NzI0ODg0MDMzMiwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjcxLjczOTU0NzcyOTQ5MjE5LCJsZWZ0Ijo4LjU2OTczODM4ODA2MTUyMywid2lkdGgiOjQwLjM0OTY5MzI5ODMzOTg0NCwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjczLjMyMDA5MTI0NzU1ODYsImxlZnQiOjguNTY4MDU5OTIxMjY0NjQ4LCJ3aWR0aCI6NDAuMDIwMDg4MTk1ODAwNzgsImhlaWdodCI6MS41MTk4NTUyNjA4NDg5OTl9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo3NC45MDA2NDIzOTUwMTk1MywibGVmdCI6OC41NjgwNTk5MjEyNjQ2NDgsIndpZHRoIjozOS45OTk5Mjc1MjA3NTE5NSwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjc2LjQ4MTE4NTkxMzA4NTk0LCJsZWZ0Ijo4LjU2ODA1NzA2MDI0MTcsIndpZHRoIjo0MC4zNTcwNzQ3Mzc1NDg4MywiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjc4LjA2MTcyOTQzMTE1MjM0LCJsZWZ0Ijo4LjU2OTczNjQ4MDcxMjg5LCJ3aWR0aCI6NDAuMzUzNzEzOTg5MjU3ODEsImhlaWdodCI6MS41MTk4NTUyNjA4NDg5OTl9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2MC42NzU3Mjc4NDQyMzgyOCwibGVmdCI6NTEuNDI3MTY1OTg1MTA3NDIsIndpZHRoIjo0MC4zNzg1NjY3NDE5NDMzNiwiaGVpZ2h0IjoxOC45MDU4NTUxNzg4MzMwMDh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo4MS45MjczMzc2NDY0ODQzOCwibGVmdCI6OC40MDE0OTc4NDA4ODEzNDgsIndpZHRoIjo3My45NTQ2OTY2NTUyNzM0NCwiaGVpZ2h0IjoyLjY3ODc2MjY3NDMzMTY2NX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEifX1d. These disruptions exacerbate rural poverty, pushing fishers toward unsustainable practices like overharvesting juveniles.

Local agriculture bears indirect yet profound burdens from HAB-contaminated irrigation waters. Peri-urban lakes supply 30-50% of irrigation needs in states like Karnataka and Telangana, but cyanotoxin-laden runoff bioaccumulates in crops, rendering produce unsafe(Gobler et al., 2024; Meerhoff et al., 2022)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik0uIiwiZmFtaWx5IjoiTWVlcmhvZmYifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSi4iLCJmYW1pbHkiOiJBdWRldCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJULkEuIiwiZmFtaWx5IjoiRGF2aWRzb24ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTC4gRGUiLCJmYW1pbHkiOiJNZWVzdGVyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlMuIiwiZmFtaWx5IjoiSGlsdCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTLiIsImZhbWlseSI6Iktvc3RlbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJaYWlodWEiLCJmYW1pbHkiOiJMaXUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTi4iLCJmYW1pbHkiOiJNYXp6ZW8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSGFucyBXLiIsImZhbWlseSI6IlBhZXJsIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik0uIiwiZmFtaWx5IjoiU2NoZWZmZXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiRS4iLCJmYW1pbHkiOiJKZXBwZXNlbiJ9fV0sImlkcyI6eyJpc3NuIjpbXSwiaXNibiI6W10sImRvaSI6IjEwLjE3NjE1Lzd2ZTUtdzAxNiIsIm9wZW5hbGV4IjoiVzQzMTg0MTU1NzcifSwidGl0bGUiOiJGZWVkYmFjayBiZXR3ZWVuIGNsaW1hdGUgY2hhbmdlIGFuZCBldXRyb3BoaWNhdGlvbjogcmV2aXNpdGluZyB0aGUgYWxsaWVkIGF0dGFjayBjb25jZXB0IGFuZCBob3cgdG8gc3RyaWtlIGJhY2siLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTc2MTUvN3ZlNS13MDE2IiwicmVzb3VyY2VVcmwiOiJodHRwczovL2Nkci5saWIudW5jLmVkdS9kb3dubG9hZHMvaDk4OXJjOTM4IiwiaXNzdWVkRGF0ZSI6IjIwMjItMDEtMDEiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTEwIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJDYXJvbGluYSBEaWdpdGFsIFJlcG9zaXRvcnkgKFVuaXZlcnNpdHkgb2YgTm9ydGggQ2Fyb2xpbmEgYXQgQ2hhcGVsIEhpbGwpIiwiaW1wYWN0RmFjdG9yIjoiMC4xMCJ9LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJVbml2ZXJzaXR5IG9mIE5vcnRoIENhcm9saW5hIGF0IENoYXBlbCBIaWxsIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjowLCJjaXRlZEJ5Q291bnQiOjN9fSwicXVvdGUiOiJXaXRoIGNsaW1hdGUgd2FybWluZywgdGhlIGRpcmVjdCBhbmQgaW5kaS1yZWN0IGVmZmVjdHMgb2YgaW5jcmVhc2VkIGxha2UgdGVtcGVyYXR1cmVzIHdpbGwgbGlrZWx5IGRyaXZlIGNoYW5nZXMgaW4gY3lhbm9iYWN0ZXJpYWwgdGF4YSBhbmQgdGhlaXIgcHJvZHVjLXRpb24gb2YgY3lhbm9iYWN0ZXJpYWwgdG94aW5zLCBsaWtlbHkgaW5jbHVkaW5nIHByb21vdC1pbmcgdGhlIHNlbGVjdGlvbiBvZiBhIGZldyBNaWNyb2N5c3RpcyBzcGVjaWVzIG9yIHN0cmFpbnMgcHJvZHVjaW5nIGhpZ2hseSB0b3hpYyBtaWNyb2N5c3Rpbi1MUiAoTWFudHpvdWtpIGV0IGFsLiAyMDE4KS4gRGVzcGl0ZSB2YXJpYXRpb25zIGFtb25nIGxha2VzIGluIGEgd2F0ZXItc2NhcGUsIGEgcmVnaW9uYWwgaW5jcmVhc2UgaW4gbWljcm9jeXN0aW4gbWF4aW1hIGFuZCBkdXJhdGlvbiBvZiBibG9vbXMgaGFzIGFsc28gYmVlbiBjbGVhcmx5IGV4cGxhaW5lZCBieSBpbmNyZWFzaW5nIHRlbXBlcmF0dXJlIGluIGNlbnRyYWwgTm9ydGggQW1lcmljYSAoSGF5ZXMgZXQgYWwuIDIwMjApLCB3aXRoIGluY3JlYXNpbmcgcmlza3MgdG8gaHVtYW4gaGVhbHRoLiBHcm93aW5nIGV2aWRlbmNlIGluZGljYXRlcyB0aGF0IGN5YW5vYmFjdGVyaWFsIGRvbWluYW5jZSB3aWxsIGV4ZXJ0IGEgcG9zaXRpdmUgZmVlZGJhY2sgd2l0aCBjbGltYXRlIHdhcm1pbmcgKEhhdmVucyBldCBhbC4gMjAxNiwgUGFlcmwgZXQgYWwuIDIwMjApLiBNYW55IGN5YW5vYmFjdGVyaWFsIHNwZWNpZXMgcHJvbGlmZXJhdGUgb24gdGhlIGxha2Ugc3VyZmFjZSwgYWJzb3JiaW5nIGxpZ2h0LCBlbmhhbmNpbmcgbG9jYWxpemVkIHRlbXBlcmF0dXJlIGluY3JlYXNlcywgYW5kIGluY3JlYXNpbmcgdGhlcm1hbCBzdHJhdGlmaWNhdGlvbiBwb3RlbnRpYWwgKEthaHJ1IGV0IGFsLiAxOTkzKS4gQ3lhbm9iYWN0ZXJpYSBwcm9tb3RlIHRoZSByZWxlYXNlIG9mIEdIR3MsIHBhcnRpY3VsYXJseSBDSDQgKFlhbiBldCBhbC4gMjAxNyksIGR1cmluZyB0aGUgZGVjb21wb3NpdGlvbiBmb2xsb3dpbmcgdGhlIGNvbGxhcHNlIG9mIGEgYmxvb20uIEN5YW5vYmFjdGVyaWEgYWxzbyBwcm9tb3RlIHJlbWluZXJhbGl6YXRpb24gb2YgbnV0cmktZW50cyBkdXJpbmcgYmxvb21zLCBsZWFkaW5nIHRvIG11bHRpcGxlIGJsb29tcyBpbiBhIHNpbmdsZSBzZWFzb24uIFVuZGVyIGV1dHJvcGhpYyBhbmQgd2FybWluZyBjb25kaS10aW9ucywgY3lhbm9iYWN0ZXJpYSBkb21pbmFuY2UgbWF5IHRodXMgYmUgYSBzZWxmLXBlcnBldHVhdGluZyBwcm9waGVjeSAoRmlnLiAyIGFuZCAzKS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo2Ny4wNjM0MDAyNjg1NTQ2OSwibGVmdCI6NTEuNDc4ODAxNzI3Mjk0OTIsIndpZHRoIjozOC42NzM0MjM3NjcwODk4NDQsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6NjguNjk4NTQ3MzYzMjgxMjUsImxlZnQiOjUxLjQ3ODgwMTcyNzI5NDkyLCJ3aWR0aCI6MzguNjc5NDk2NzY1MTM2NzIsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6NzAuMzMzNjg2ODI4NjEzMjgsImxlZnQiOjUxLjQ3ODgwMTcyNzI5NDkyLCJ3aWR0aCI6MzguNjc3NTgxNzg3MTA5Mzc1LCJoZWlnaHQiOjEuNDcxMTAxNTIyNDQ1Njc4N319LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjcxLjk2ODgyNjI5Mzk0NTMxLCJsZWZ0Ijo1MS40Nzg4MDE3MjcyOTQ5Miwid2lkdGgiOjM4LjY3OTEwMzg1MTMxODM2LCJoZWlnaHQiOjEuNDcxMTAxNTIyNDQ1Njc4N319LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjczLjYxMDU3MjgxNDk0MTQsImxlZnQiOjUxLjQ3ODgwMTcyNzI5NDkyLCJ3aWR0aCI6MzguNjc0NDI3MDMyNDcwNywiaGVpZ2h0IjoxLjQ3Mzc0Njg5NTc5MDF9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo3NS4yNDcwMzk3OTQ5MjE4OCwibGVmdCI6NTEuNDc4ODA5MzU2Njg5NDUsIndpZHRoIjozOC42NzQyNjMwMDA0ODgyOCwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo3Ni44ODIxODY4ODk2NDg0NCwibGVmdCI6NTEuNDc4ODA5MzU2Njg5NDUsIndpZHRoIjozOC42ODExNzE0MTcyMzYzMywiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo3OC41MjUyNTMyOTU4OTg0NCwibGVmdCI6NTEuNDc4ODA5MzU2Njg5NDUsIndpZHRoIjozOC42NzU2NjI5OTQzODQ3NjYsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODAuMTYwNDAwMzkwNjI1LCJsZWZ0Ijo1MS40Nzg4MDkzNTY2ODk0NSwid2lkdGgiOjM4LjY4MjAyOTcyNDEyMTA5NCwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo4MS43OTU1Mzk4NTU5NTcwMywibGVmdCI6NTEuNDc4ODA5MzU2Njg5NDUsIndpZHRoIjozOC42NzQ5ODM5NzgyNzE0ODQsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODMuNDM4NjEzODkxNjAxNTYsImxlZnQiOjUxLjQ3ODgwOTM1NjY4OTQ1LCJ3aWR0aCI6MzguNjc2MTg1NjA3OTEwMTU2LCJoZWlnaHQiOjEuNDg2OTczNTI0MDkzNjI4fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODUuMDg5NjMwMTI2OTUzMTIsImxlZnQiOjUxLjQ3ODgwOTM1NjY4OTQ1LCJ3aWR0aCI6NC42ODI4MDUwNjEzNDAzMzIsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODYuNzI0Nzc3MjIxNjc5NjksImxlZnQiOjUzLjQ0MTE3NzM2ODE2NDA2LCJ3aWR0aCI6Ni4xNDAzNTU1ODcwMDU2MTUsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODYuNzI0ODkxNjYyNTk3NjYsImxlZnQiOjYwLjk5NDg3Njg2MTU3MjI2Niwid2lkdGgiOjYuMDM2OTkyNTQ5ODk2MjQsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODYuNzI0ODkxNjYyNTk3NjYsImxlZnQiOjY4LjQ0NjUyNTU3MzczMDQ3LCJ3aWR0aCI6Ni4xNTA3MDU4MTQzNjE1NzIsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODYuNzI0ODkxNjYyNTk3NjYsImxlZnQiOjc2LjAwOTgyNjY2MDE1NjI1LCJ3aWR0aCI6Mi43MjczMzMzMDcyNjYyMzU0LCJoZWlnaHQiOjEuNDcxMTAxNTIyNDQ1Njc4N319LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjg2LjcyNDg5MTY2MjU5NzY2LCJsZWZ0Ijo4MC4xNDk1ODE5MDkxNzk2OSwid2lkdGgiOjEwLjAwODIzNDk3NzcyMjE2OCwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo4OC4zNjAwMzg3NTczMjQyMiwibGVmdCI6NTEuNDc3NjU3MzE4MTE1MjM0LCJ3aWR0aCI6MzguNjcyMDU0MjkwNzcxNDg0LCJoZWlnaHQiOjEuNDcxMTAxNTIyNDQ1Njc4N319LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjkwLjAwMzExMjc5Mjk2ODc1LCJsZWZ0Ijo1MS40Nzc2NTczMTgxMTUyMzQsIndpZHRoIjozOC42NzY0OTg0MTMwODU5NCwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo5MS42MzgyNTIyNTgzMDA3OCwibGVmdCI6NTEuNDc3NjUzNTAzNDE3OTcsIndpZHRoIjozOC42NzU4MzQ2NTU3NjE3MiwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjksImJib3giOnsidG9wIjo2LjQ2ODkzNTk2NjQ5MTY5OSwibGVmdCI6OS44NDAxMzM2NjY5OTIxODgsIndpZHRoIjozOC42ODYzMjUwNzMyNDIxOSwiaGVpZ2h0IjoxNi4yMTIxMjU3NzgxOTgyNDJ9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI5In19LHsib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkNocmlzdG9waGVyIEouIiwiZmFtaWx5IjoiR29ibGVyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlJ1dGggVy4iLCJmYW1pbHkiOiJEcmlua3dhdGVyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkEuIFcuIiwiZmFtaWx5IjoiQW50aG9ueSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJKZW5uaWZlciBBLiIsImZhbWlseSI6IkdvbGVza2kifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW5uIE1hcmllIEUuIiwiZmFtaWx5IjoiRmFtdWxhcm8tUGVjb3JhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1pY2hhZWwgQi4iLCJmYW1pbHkiOiJXYWxsYWNlIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5vcmEgUi4gVy4iLCJmYW1pbHkiOiJTdHJhcXVhZGluZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJSb25vam95IiwiZmFtaWx5IjoiSGVtIn19XSwiaWRzIjp7Imlzc24iOlsiMTY2NC0zMDJYIl0sImlzYm4iOltdLCJkb2kiOiIxMC4zMzg5L2ZtaWNiLjIwMjQuMTQ2NDY4NiIsInBtaWQiOiIzOTU2ODk5NyIsIm9wZW5hbGV4IjoiVzQ0MDQxMjA5NTIiLCJleGEiOiJodHRwczovL3d3dy5mcm9udGllcnNpbi5vcmcvam91cm5hbHMvbWljcm9iaW9sb2d5L2FydGljbGVzLzEwLjMzODkvZm1pY2IuMjAyNC4xNDY0Njg2L2Z1bGwifSwidGl0bGUiOiJTZXdhZ2UtYW5kIGZlcnRpbGl6ZXItZGVyaXZlZCBudXRyaWVudHMgYWx0ZXIgdGhlIGludGVuc2l0eSwgZGl2ZXJzaXR5LCBhbmQgdG94aWNpdHkgb2YgaGFybWZ1bCBjeWFub2JhY3RlcmlhbCBibG9vbXMgaW4gZXV0cm9waGljIGxha2VzIiwiYWJzdHJhY3QiOiJDeWFub2JhY3RlcmlhbCBoYXJtZnVsIGFsZ2FsIGJsb29tcyAoQ0hBQnMpIGFyZSBwcm9tb3RlZCBieSBleGNlc3NpdmUgbnV0cmllbnQgbG9hZGluZyBhbmQsIHdoaWxlIGZlcnRpbGl6ZXJzIGFuZCBzZXdhZ2UgYXJlIHRoZSBtb3N0IHByZXZhbGVudCBleHRlcm5hbCBudXRyaWVudCBzb3VyY2VzIGluIG1vc3Qgd2F0ZXJzaGVkcywgdGhlIGRpZmZlcmVudGlhbCBlZmZlY3RzIG9mIHRoZXNlIG51dHJpZW50IHNvdXJjZXMgb24gQ0hBQnMgYXJlIHVua25vd24uIEhlcmUsIHdlIHRyYWNrZWQgQ0hBQnMgYW5kIHBlcmZvcm1lZCBleHBlcmltZW50cyBpbiBmaXZlIGRpc3RpbmN0IGxha2VzIGFjcm9zcyB0aGUgTm9ydGhlcm4gVVMgaW5jbHVkaW5nIExha2UgRXJpZS4gRmVydGlsaXplcnMgd2l0aCBhbW1vbml1bSBhbmQgb3J0aG9waG9zcGhhdGUsIG1lbWJyYW5lICgwLjIgzrxtKS1maWx0ZXJlZCBzZXdhZ2UgKGRvbWluYXRlZCBieSByZWR1Y2VkIGZvcm1zIG9mIG5pdHJvZ2VuKSBzYW5kLWFuZCBtZW1icmFuZS1maWx0ZXJlZCBzZXdhZ2UgKGRvbWluYXRlZCBieSBuaXRyYXRlKSwgYW5kIGFuIGlub3JnYW5pYyBudXRyaWVudCBzb2x1dGlvbiBvZiBhbW1vbml1bSBhbmQgb3J0aG9waG9zcGhhdGUgd2VyZSB1c2VkIGFzIGV4cGVyaW1lbnRhbCBudXRyaWVudCBzb3VyY2VzIGZvciBDSEFCcyBhdCBOLWVxdWl2YWxlbnQsIGVudmlyb25tZW50YWxseSByZWFsaXN0aWMgY29uY2VudHJhdGlvbnMuIFBoeXRvcGxhbmt0b24gY29tbXVuaXRpZXMgd2VyZSBldmFsdWF0ZWQgZmx1b3JvbWV0cmljYWxseSwgbWljcm9zY29waWNhbGx5LCBhbmQgdmlhIGhpZ2ggdGhyb3VnaHB1dCBzZXF1ZW5jaW5nIG9mIHRoZSAxNlMgclJOQSBnZW5lLCBhbmQgbGV2ZWxzIG9mIG1pY3JvY3lzdGluIGFuZCB0aGUgzrQgMTUgTiBjb250ZW50IG9mIHBhcnRpY3VsYXRlIG9yZ2FuaWMgbml0cm9nZW4gKM60UE8gMTUgTikgd2VyZSBxdWFudGlmaWVkLiBGZXJ0aWxpemVyIGFuZCBib3RoIHNvdXJjZXMgb2Ygd2FzdGV3YXRlciBpbmNyZWFzZWQgdGhlIGFidW5kYW5jZSBvZiBjeWFub2JhY3RlcmlhIGluIGFsbCBleHBlcmltZW50cyBhY3Jvc3MgYWxsIGZpdmUgbGFrZXMgKCBwICZhbXA7bHQ7IDAuMDUgZm9yIGFsbCkgd2hlcmVhcyBlZmZlY3RzIG9uIGV1a2FyeW90aWMgcGh5dG9wbGFua3RvbiB3ZXJlIGxpbWl0ZWQuIFNhbmQtZmlsdGVyZWQgc2V3YWdlIGNvbnRhaW5lZCBsZXNzIFAsIG9yZ2FuaWMgbWF0dGVyLCBhbmQgYW1tb25pdW0gYnV0IG1vcmUgbml0cmF0ZSBhbmQgaGFkIGEgMjUlIGxlc3MgcG90ZW50IHN0aW11bGF0b3J5IGVmZmVjdCBvbiBjeWFub2JhY3RlcmlhIHRoYW4gbWVtYnJhbmUtZmlsdGVyZWQgc2V3YWdlLCBzdWdnZXN0aW5nIG5pdHJpZmljYXRpb24gbWF5IHBsYXkgYSByb2xlIGluIHJlZHVjaW5nIENIQUJzLiBGZXJ0aWxpemVyIGluY3JlYXNlZCBtaWNyb2N5c3RpbiBsZXZlbHMgYW5kIGRlY3JlYXNlZCB0aGUgzrRQTyAxNSBOIHdoZXJlYXMgd2FzdGV3YXRlciBpbmNyZWFzZWQgzrRQTyAxNSBOICggcCAmYW1wO2x0OyAwLjA1IGZvciBhbGwpLiBNaWNyb2N5c3RpcyB3YXMgdGhlIGdlbnVzIG1vc3QgY29uc2lzdGVudGx5IHByb21vdGVkIGJ5IG51dHJpZW50IHNvdXJjZXMgKCBwICZhbXA7bHQ7IDAuMDUgaW4gYWxsIGV4cGVyaW1lbnRzKSwgZm9sbG93ZWQgYnkgQ3lhbm9iaXVtICggcCAmYW1wO2x0OyAwLjA1IGluIDUwJSBvZiBleHBlcmltZW50cyksIHdpdGggaW5jcmVhc2VzIGluIE1pY3JvY3lzdGlzIGJpb21hc3MgY29uc2lzdGVudGx5IGVsaWNpdGVkIGJ5IG1lbWJyYW5lLWZpbHRlcmVkIHdhc3Rld2F0ZXIuIENvbGxlY3RpdmVseSwgcmVzdWx0cyBkZW1vbnN0cmF0ZSB0aGF0IGRpZmZlcmluZyB0eXBlcyBvZiBzZXdhZ2UgZGlzY2hhcmdlIGFuZCBmZXJ0aWxpemVycyBjYW4gcHJvbW90ZSBDSEFCIGludGVuc2l0eSBhbmQgdG94aWNpdHksIHdoaWxlIGNvbmN1cnJlbnRseSBhbHRlcmluZyBDSEFCIGRpdmVyc2l0eSBhbmQgzrRQTyAxNSBOLiBXaGlsZSBtZW1icmFuZS1maWx0ZXJlZCBzZXdhZ2UgY29uc2lzdGVudGx5IGZhdm9yZWQgTWljcm9jeXN0aXMgLCB0aGUgZGlzY2hhcmdlIG9mIHNld2FnZSB0aHJvdWdoIHNhbmRzIG11dGVkIGJsb29tIGludGVuc2l0eSBzdWdnZXN0aW5nIHNhbmQtYmVkcyBtYXkgcmVwcmVzZW50IGEgdG9vbCB0byByZW1vdmUga2V5IG51dHJpZW50cyBhbmQgcGFydGlhbGx5IG1pdGlnYXRlIENIQUJzLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzg5L2ZtaWNiLjIwMjQuMTQ2NDY4NiIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cuZnJvbnRpZXJzaW4ub3JnL2pvdXJuYWxzL21pY3JvYmlvbG9neS9hcnRpY2xlcy8xMC4zMzg5L2ZtaWNiLjIwMjQuMTQ2NDY4Ni9wZGYiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMS0wNiIsImFjY2Vzc2VkRGF0ZSI6IjIwMjYtMDIiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZyb250aWVycyBpbiBNaWNyb2Jpb2xvZ3kiLCJ2b2x1bWUiOjE1LCJpbXBhY3RGYWN0b3IiOiIzLjk5In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTQ2NDY4NiwibGFzdFBhZ2UiOjE0NjQ2ODZ9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRnJvbnRpZXJzIE1lZGlhIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo3MywiY2l0ZWRCeUNvdW50Ijo5fX0sInF1b3RlIjoiZGVyaXZlZCBudXRyaWVudHMgYWx0ZXIgdGhlIGludGVuc2l0eSwgZGl2ZXJzaXR5LCBhbmQgdG94aWNpdHkgb2YgaGFybWZ1bCBjeWFub2JhY3RlcmlhbCBibG9vbXMgLi4uIEN5YW5vYmFjdGVyaWFsIGhhcm1mdWwgYWxnYWwgYmxvb21zIChDSEFCcykgYXJlIHByb21vdGVkIGJ5IGV4Y2Vzc2l2ZSBudXRyaWVudCBsb2FkaW5nIGFuZCwgd2hpbGUgZmVydGlsaXplcnMgYW5kIHNld2FnZSBhcmUgdGhlIG1vc3QgcHJldmFsZW50IGV4dGVybmFsIG51dHJpZW50IHNvdXJjZXMgaW4gbW9zdCB3YXRlcnNoZWRzLCB0aGUgZGlmZmVyZW50aWFsIGVmZmVjdHMgb2YgdGhlc2UgbnV0cmllbnQgLi4uIG9yZ2FuaWMgbml0cm9nZW4gKM60UE8xNU4pIHdlcmUgcXVhbnRpZmllZC4gRmVydGlsaXplciBhbmQgYm90aCBzb3VyY2VzIG9mIHdhc3Rld2F0ZXIgaW5jcmVhc2VkIHRoZSBhYnVuZGFuY2Ugb2YgLi4uIHBoeXRvcGxhbmt0b24gd2VyZSBsaW1pdGVkLiBTYW5kLWZpbHRlcmVkIHNld2FnZSBjb250YWluZWQgbGVzcyBQLCBvcmdhbmljIG1hdHRlciwgYW5kIGFtbW9uaXVtIGJ1dCBtb3JlIG5pdHJhdGUgYW5kIC4uLiBkaWZmZXJpbmcgdHlwZXMgb2Ygc2V3YWdlIGRpc2NoYXJnZSBhbmQgZmVydGlsaXplcnMgY2FuIHByb21vdGUgQ0hBQiBpbnRlbnNpdHkgYW5kIHRveGljaXR5LCB3aGlsZSBjb25jdXJyZW50bHkgLi4uIHNld2FnZSB0aHJvdWdoIHNhbmRzIG11dGVkIGJsb29tIGludGVuc2l0eSBzdWdnZXN0aW5nIHNhbmQtYmVkcyBtYXkgcmVwcmVzZW50IGEgdG9vbCB0byByZW1vdmUga2V5IG51dHJpZW50cyBhbmQgLi4uICggV2F0c29uIGV0IGFsLiwgMjAxNiA7IEJ1bGxlcmphaG4gZXQgYWwuLCAyMDE2IDsgSHVpc21hbiBldCBhbC4sIDIwMTggKSBhbmQgZXhjZXNzaXZlIG51dHJpZW50IGxvYWRpbmcgaXMgdGhlIHByb2Nlc3MgbW9zdCBjb21tb25seSBjaXRlZCBhcyB0aGUgZW52aXJvbm1lbnRhbCBmYWN0b3IgcHJvbW90aW5nIENIQUJzICgifV0%3D. Microcystins detected in irrigated vegetables from Hussainsagar reached 5-20 μg/kg, exceeding safety thresholds and triggering export rejections(Wang et al., 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ilhpd2VuIiwiZmFtaWx5IjoiV2FuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLdW4iLCJmYW1pbHkiOiJTaGkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiWXVubGluIiwiZmFtaWx5IjoiWmhhbmcifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQm9xaWFuZyIsImZhbWlseSI6IlFpbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJZaWJvIiwiZmFtaWx5IjoiWmhhbmcifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiV2VpamlhIiwiZmFtaWx5IjoiV2FuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJSLiBJZXN0eW4iLCJmYW1pbHkiOiJXb29sd2F5In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNoaWxvbmciLCJmYW1pbHkiOiJQaWFvIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkVyaWsiLCJmYW1pbHkiOiJKZXBwZXNlbiJ9fV0sImlkcyI6eyJpc3NuIjpbXSwiaXNibiI6W10sImRvaSI6IjEwLjIxMjAzL3JzLjMucnMtMjQ0MjE0Mi92MSIsIm9wZW5hbGV4IjoiVzQzMTcwMzg4MjcifSwidGl0bGUiOiJMYWtlIHdhcm1pbmcgYW5kIGluY3JlYXNlZCBoZWF0d2F2ZXMgcmV2ZWFsZWQgYnkgYSBub3ZlbCBkYXRhLWRyaXZlbiBtb2RlbGluZyBhcHByb2FjaCIsImFic3RyYWN0IjoiPHRpdGxlPkFic3RyYWN0PC90aXRsZT4gQ2xpbWF0ZSBjaGFuZ2UgY291bGQgc2VyaW91c2x5IHRocmVhdGVuIGdsb2JhbCBsYWtlIGVjb3N5c3RlbXMgYnkgd2FybWluZyBsYWtlIHN1cmZhY2Ugd2F0ZXIgYW5kIGluY3JlYXNpbmcgdGhlIG9jY3VycmVuY2Ugb2YgbGFrZSBoZWF0d2F2ZXMuIFlldCwgdGhlcmUgYXJlIGdyZWF0IHVuY2VydGFpbnRpZXMgaW4gcXVhbnRpZnlpbmcgbGFrZSB0ZW1wZXJhdHVyZSBjaGFuZ2VzIGdsb2JhbGx5IGR1ZSB0byBsYWNrIG9mIGFjY3VyYXRlIGxhcmdlLXNjYWxlIG1vZGVsIHNpbXVsYXRpb25zLiBIZXJlLCB3ZSBpbnRlZ3JhdGVkIHNhdGVsbGl0ZSBvYnNlcnZhdGlvbnMgYW5kIGEgbnVtZXJpY2FsIG1vZGVsIHRvIGltcHJvdmUgbGFrZSB0ZW1wZXJhdHVyZSBtb2RlbGxpbmcgYW5kIHRvIGV4cGxvcmUgdGhlIG11bHRpZmFjZXRlZCBjaGFyYWN0ZXJpc3RpY3Mgb2YgdHJlbmRzIGluIHN1cmZhY2UgdGVtcGVyYXR1cmVzIGFuZCBsYWtlIGhlYXR3YXZlIG9jY3VycmVuY2UgaW4gQ2hpbmVzZSBsYWtlcyBmcm9tIDE5ODAgdG8gMjEwMC4gT3VyIG1vZGVsLWRhdGEgaW50ZWdyYXRpb24gYXBwcm9hY2ggcmV2ZWFsZWQgdGhhdCB0aGUgbGFrZSBzdXJmYWNlIHdhdGVycyBoYXZlIHdhcm1lZCBhdCBhIHJhdGUgb2YgMC4xMSDCsEMgZGVjYWRlLTEgZHVyaW5nIHRoZSBwZXJpb2QgMTk4MC0yMDIxLCBiZWluZyBvbmx5IGhhbGYgb2YgdGhlIHB1cmUgbW9kZWwtYmFzZWQgZXN0aW1hdGUuIE1vcmVvdmVyLCBvdXIgYW5hbHlzaXMgc3VnZ2VzdGVkIHRoYXQgYW4gYXN5bW1ldHJpYyBzZWFzb25hbCB3YXJtaW5nIHJhdGUgaGFzIGxlZCB0byBhIHJlZHVjZWQgdGVtcGVyYXR1cmUgc2Vhc29uYWxpdHkgaW4gZWFzdGVybiBwbGFpbiBsYWtlcyBidXQgYW4gYW1wbGlmaWVkIG9uZSBpbiBhbHBpbmUgbGFrZXMuIFRoZSBkdXJhdGlvbnMgb2YgbGFrZSBoZWF0d2F2ZXMgaGF2ZSBhbHNvIGluY3JlYXNlZCBhdCBhIHJhdGUgb2YgNy43IGRheXMgZGVjYWRlLTEuIFVuZGVyIHRoZSBoaWdoLWdyZWVuaG91c2UtZ2FzLWVtaXNzaW9uIHNjZW5hcmlvLCBsYWtlIHN1cmZhY2UgdGVtcGVyYXR1cmUgYW5kIGxha2UgaGVhdHdhdmUgZHVyYXRpb24gd2VyZSBwcm9qZWN0ZWQgdG8gaW5jcmVhc2UgYnkgMi4yIMKwQyBhbmQgMTk3IGRheXMgYXQgdGhlIGVuZCBvZiAyMXN0IGNlbnR1cnksIHJlc3BlY3RpdmVseS4gU3VjaCBkcmFzdGljIGNoYW5nZXMgd291bGQgd29yc2VuIHRoZSBlbnZpcm9ubWVudGFsIGNvbmRpdGlvbnMgb2YgbGFrZXMgc3ViamVjdGVkIHRvIGhpZ2ggYW5kIGluY3JlYXNpbmcgYW50aHJvcG9nZW5pYyBwcmVzc3VyZXMsIHBvc2luZyBncmVhdCB0aHJlYXRzIHRvIGFxdWF0aWMgYmlvZGl2ZXJzaXR5IGFuZCBodW1hbiBoZWFsdGguIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjIxMjAzL3JzLjMucnMtMjQ0MjE0Mi92MSIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cucmVzZWFyY2hzcXVhcmUuY29tL2FydGljbGUvcnMtMjQ0MjE0Mi9sYXRlc3QucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjMtMDEtMTciLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTEwIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJSZXNlYXJjaCBTcXVhcmUgKFJlc2VhcmNoIFNxdWFyZSkiLCJpbXBhY3RGYWN0b3IiOiIwLjQwIn0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlJlc2VhcmNoIFNxdWFyZSAoVW5pdGVkIFN0YXRlcykifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjU3LCJjaXRlZEJ5Q291bnQiOjN9fSwicXVvdGUiOiJJZiB0aGUgYm90dG9tIHdhdGVyIHRlbXBlcmF0dXJlIGFsc28gaW5jcmVhc2VzLCBtaW5lcmFsaXphdGlvbiBvZiBvcmdhbmljIG1hdHRlciB3aWxsIGJlIGFjY2VsZXJhdGVkLCB3aGljaCB3aWxsIGxlYWQgdG8gaGlnaGVyIGVtaXNzaW9uIG9mIGNhcmJvbiBkaW94aWRlIGFuZCBtZXRoYW5lNDYsNDcsIHJlaW5mb3JjaW5nIGNsaW1hdGUgd2FybWluZyBkdWUgdG8gZ3JlZW5ob3VzZSBnYXNlcy4gVGhpcyBwcm9jZXNzIHdpbGwgc3BlZWQgdXAgdGhlIHJlY3ljbGluZyBvZiBudXRyaWVudHMgYW5kIGZhdm9yIHBoeXRvcGxhbmt0b24gZ3Jvd3RoLCBzdWdnZXN0aW5nIHBvc2l0aXZlIGZlZWRiYWNrIGJldHdlZW4gY2xpbWF0ZSBjaGFuZ2UgYW5kIGV1dHJvcGhpY2F0aW9uNDguIFRvZ2V0aGVyIHdpdGggdGhlIGhlYXZ5IGV4dGVybmFsIG51dHJpZW50IGxvYWRpbmcgZnJvbSBkb21lc3RpYywgYWdyaWN1bHR1cmFsLCBhbmQgaW5kdXN0cmlhbCB3YXN0ZSwgdGhlIGdyb3d0aCBvZiBwaHl0b3BsYW5rdG9uIGlzIHByb21vdGVkIGJ5IGhvdCB3YXRlciB1bmRlciBudXRyaWVudC1yaWNoIGNvbmRpdGlvbnM0OS4gQmVzaWRlcyB0aGF0LCBzdHJvbmdlciBzdHJhdGnvv71jYXRpb24gd2lsbCBwZXJtaXQgcGh5dG9wbGFua3RvbiB0byBjaXJjdWxhdGUgaW4gdGhlIGV1cGhvdGljIHpvbmU1MC4gSW5jcmVhc2luZyBhbGdhZSBibG9vbXMgaGF2ZSBiZWVuIHJlcG9ydGVkIHdvcmxkd2lkZSwgZXNwZWNpYWxseSBpbiByZWdpb25zIHRoYXQgZGVwZW5kIGhpZ2hseSBvbiBmZXJ0aWxpemVyczUxLiBJbiBDaGluYSwgdGhlIG9jY3VycmVuY2Ugb2YgYWxnYWUgYmxvb21zIGluIHRoZSAyMDEwcyBoYXMgaW5jcmVhc2VkIG5lYXJseSBuaW5lIHRpbWVzIHJlbGF0aXZlIHRvIHRoZSAxOTgwcy0xOTkwczUxLCBhbmQgbWFueSBibG9vbXMgaGF2ZSBiZWVuIHJlcG9ydGVkIHJlY2VudGx5IGluIG5hdHVyYWwgbGFrZXMgYW5kIHJlc2Vydm9pcnMgKFN1cHBsZW1lbnRhcnkgRmlnLiA3KS4gRnVydGhlcm1vcmUsIHRoZSB3YXJtZXIgc3VyZmFjZSB3YXRlciBhbmQgc3Ryb25nZXIgc3RyYXRp77%2B9Y2F0aW9uIHdpbGwgZmF2b3IgdGhlIGRvbWluYW5jZSBvZiBjeWFub2JhY3RlcmlhIGluIGxha2VzIGdpdmVuIGl0cyBoaWdoZXIgb3B0aW1hbCBncm93dGggdGVtcGVyYXR1cmUgYW5kIGFiaWxpdHkgdG8gYWRqdXN0IGJ1b3lhbmN5NDksIGxlYWRpbmcgdG8gaGlnaGVyIGZyZXF1ZW5jeSBvZiBoYXJtZnVsIGFsZ2FlIGJsb29tczQ3LiBUaGUgdG94aWMgc3Vic3RhbmNlcyBwcm9kdWNlZCBieSBjeWFub2JhY3RlcmlhIHdpbGwgdGhyZWF0ZW4gdGhlIHNlY3VyaXR5IG9mIGRyaW5raW5nIHdhdGVyIGFuZCBodW1hbiBoZWFsdGg2LDUyLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjQ1LjEyNjk2MDc1NDM5NDUzLCJsZWZ0Ijo0LjU3NTE2Mjg4NzU3MzI0Miwid2lkdGgiOjkwLjk0MTIwMDI1NjM0NzY2LCJoZWlnaHQiOjEuNzc3MTYzMzg2MzQ0OTA5N319LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjQ3LjQwMTcyMTk1NDM0NTcsImxlZnQiOjQuNTc1MTYyODg3NTczMjQyLCJ3aWR0aCI6OTAuMjA1MjQ1OTcxNjc5NjksImhlaWdodCI6MS43NzcxNjMzODYzNDQ5MDk3fX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NDkuNjQ5MTE2NTE2MTEzMjgsImxlZnQiOjQuNTc1MTYyODg3NTczMjQyLCJ3aWR0aCI6OTEuMDQzNzMxNjg5NDUzMTIsImhlaWdodCI6NC4yNjg4Njk4NzY4NjE1NzJ9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo1NC4zODgyMTc5MjYwMjUzOSwibGVmdCI6NC41NzUxNjI4ODc1NzMyNDIsIndpZHRoIjo0OC40NzYyMjY4MDY2NDA2MjUsImhlaWdodCI6MS45OTQxMDgwODA4NjM5NTI2fX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NTguMzk2NDM0NzgzOTM1NTUsImxlZnQiOjQuNTc1MTYyODg3NTczMjQyLCJ3aWR0aCI6ODcuODgyMzg1MjUzOTA2MjUsImhlaWdodCI6MS43NzcxNjMzODYzNDQ5MDk3fX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NjAuNjQzODI5MzQ1NzAzMTI1LCJsZWZ0Ijo0LjU3NTE2Mjg4NzU3MzI0Miwid2lkdGgiOjkwLjQ1MDU2MTUyMzQzNzUsImhlaWdodCI6MS45OTQxMDgwODA4NjM5NTI2fX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NjMuMTA4MTY5NTU1NjY0MDYsImxlZnQiOjQuNTc1MTYyODg3NTczMjQyLCJ3aWR0aCI6ODkuOTY3NTk3OTYxNDI1NzgsImhlaWdodCI6MS45OTQxMDgwODA4NjM5NTI2fX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NjUuNTcyNDQ4NzMwNDY4NzUsImxlZnQiOjQuNTc1MTYyODg3NTczMjQyLCJ3aWR0aCI6OTAuNTgwODg2ODQwODIwMzEsImhlaWdodCI6MS45OTQxNjk3MTIwNjY2NTA0fX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NjguMDM2Nzg4OTQwNDI5NjksImxlZnQiOjQuNTc1MTYyODg3NTczMjQyLCJ3aWR0aCI6ODkuMTkwNDM3MzE2ODk0NTMsImhlaWdodCI6Ni41NDM2OTMwNjU2NDMzMTA1fX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NzUuMDUwNjUxNTUwMjkyOTcsImxlZnQiOjQuNTc1MTYyODg3NTczMjQyLCJ3aWR0aCI6OTAuMzQ3MDY4Nzg2NjIxMSwiaGVpZ2h0IjoxLjk5NDE2OTcxMjA2NjY1MDR9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo3Ny41MTQ5OTE3NjAyNTM5LCJsZWZ0Ijo0LjU3NTE2Mjg4NzU3MzI0Miwid2lkdGgiOjkwLjIyMjQ4ODQwMzMyMDMxLCJoZWlnaHQiOjEuOTk0MTY5NzEyMDY2NjUwNH19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjc5Ljk3OTMzOTU5OTYwOTM4LCJsZWZ0Ijo0LjU3NTE2Mjg4NzU3MzI0Miwid2lkdGgiOjg1Ljk4MjEzMTk1ODAwNzgxLCJoZWlnaHQiOjEuOTk0MTY5NzEyMDY2NjUwNH19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjgifX1d. Soil salinization from algal debris and nutrient recycling further degrades fertility, slashing rice and vegetable yields by 20-30% in lake-adjacent farmlands (Ji et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkRhb2JpbiIsImZhbWlseSI6IkppIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllhbnhpbmciLCJmYW1pbHkiOiJIYW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTGlhbmdob25nIiwiZmFtaWx5IjoiTG9uZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJYaWFva2FuZyIsImZhbWlseSI6IlhpbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJIdWl5YW4iLCJmYW1pbHkiOiJYdSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTaXFpYW4iLCJmYW1pbHkiOiJRaXUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSmlhbmdodWFpIiwiZmFtaWx5IjoiTWVuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJYaW5neGluZyIsImZhbWlseSI6IlpoYW8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiWWFuYW4iLCJmYW1pbHkiOiJIdWFuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJEZWZ1IiwiZmFtaWx5IjoiTGl1In19XSwiaWRzIjp7Imlzc24iOlsiMjI5Ni03MDFYIl0sImlzYm4iOltdLCJkb2kiOiIxMC4zMzg5L2Zldm8uMjAyMy4xMjk3MDQ3Iiwib3BlbmFsZXgiOiJXNDM5MDYxMDY2OCJ9LCJ0aXRsZSI6Ikh5cG94aWEgYW5kIGl0cyBmZWVkYmFjayByZXNwb25zZSB0byBhbGdhbCBibG9vbXMgYW5kIENINCBlbWlzc2lvbnMgaW4gc3VidHJvcGljYWwgcmVzZXJ2b2lycyIsImFic3RyYWN0IjoiQW50aHJvcG9nZW5pYyBudXRyaWVudCBpbnB1dCBzdGltdWxhdGVzIHN1cmZhY2UgZXV0cm9waGljYXRpb24gYW5kIGFsZ2FsIGJsb29tcyBhbmQgdGh1cyBleGFjZXJiYXRlcyBveHlnZW4gZGVwbGV0aW9uIGFuZCBoeXBveGlhIGluIGFxdWF0aWMgZWNvc3lzdGVtcy4gSG93ZXZlciwgZXhpc3Rpbmcgc3R1ZGllcyBoYXZlIHBhaWQgbGl0dGxlIGF0dGVudGlvbiBvbiB0aGUgdW5kZXJzdGFuZGluZyBvZiB0aGUgZmVlZGJhY2sgcmVsYXRpb25zaGlwcyBhbW9uZyBveHlnZW4gY29uc3VtcHRpb24sIGFsZ2FsIHN1Y2Nlc3Npb24gYW5kIGdyZWVuaG91c2UgZ2FzIGVmZmVjdCBhZnRlciByaXZlciBkYW1taW5nLCBwYXJ0aWN1bGFybHkgaW4gVGhyZWUgR29yZ2VzIFJlc2Vydm9pci4gVGhpcyBmaWVsZCB3b3JrIGludmVzdGlnYXRlZCB0aGUgdGhlcm1hbCBzdHJhdGlmaWNhdGlvbiwgd2F0ZXIgcXVhbGl0eSBhbmQgcGh5dG9wbGFua3RvbiBpbiBmb3VyIHRyaWJ1dGFyaWVzIChQZW5neGkgQmF5IChQWEIpLCBEYW5pbmcgQmF5IChETkIpLCBTaGVubm9uZyBCYXkgKFNOQiksIGFuZCBYaWFuZ3hpIEJheSAoWFhCKSkgb2YgdGhlIFRocmVlIEdvcmdlcyBSZXNlcnZvaXIgKFRHUikgaW4gc3ByaW5nIChBcHJpbCAyNS0yOCwgMjAyMiksIGFuZCBleHBsb3JlZCB0aGUgZHJpdmVycyBvZiBoeXBveGlhIGFuZCBpdHMgZmVlZGJhY2sgcmVzcG9uc2UgdG8gYWxnYWwgYmxvb21zIGFuZCBtZXRoYW5lIChDSCA0ICkgZW1pc3Npb25zLiBUaGUgcmVzdWx0cyBzaG93ZWQgdGhhdCBkdXJpbmcgdGhlIG9ic2VydmF0aW9uIHBlcmlvZCwgRE8sIHRoZXJtYWwgc3RyYXRpZmljYXRpb24sIGFuZCBhbGdhbCBibG9vbXMgd2VyZSBtb3JlIHNldmVyZSBpbiBQWEIuIFRoZSBoaWdoIENoZW1pY2FsIE94eWdlbiBEZW1hbmQgKENPRCBNbiApICgxMS4yNyBtZ%2BKAokwgLTEgKSBhbmQgRGlzc29sdmVkIE9yZ2FuaWMgQ2FyYm9uIChET0MpIGNvbnRlbnQgKDcuNTYgbWfigKJMIC0xICkgaW4gUFhCIHdlcmUgdGhlIG1haW4gY2F1c2VzIG9mIHRoZSBwcm9ncmVzc2l2ZSBoeXBveGlhLiBGdXJ0aGVybW9yZSwgc3Ryb25nZXIgZGVuc2l0eSBzdHJhdGlmaWNhdGlvbiBpbiBQWEIgaW5oaWJpdGVkIHRoZSB2ZXJ0aWNhbCBzdXBwbHkgb2YgRE8gYW5kIGFjY2VsZXJhdGVkIGJvdHRvbSBoeXBveGlhLiBCdXQgdGhlIGRvd25zbG9wZSBkZW5zaXR5IGN1cnJlbnQgcmVwbGVuaXNoZWQgYm90dG9tIERPIGluIHRoZSB1cHBlciByZWFjaGVzIG9mIHRoZSBETkIsIFNOQiwgYW5kIFhYQi4gVGhlIENINCBjb25jZW50cmF0aW9uIHdhcyBnZW5lcmFsbHkgaGlnaGVyIGluIGh5cG94aWEgem9uZSwgZXNwZWNpYWxseSBpbiB3aGVyZSBjeWFub2JhY3RlcmlhIGFyZSB0aGUgZG9taW5hbnQgc3BlY2llcyAoZWcuLCBQWDAzIGFuZCBQWDA0KS4gRGFtIGNvbnN0cnVjdGlvbiBsZWQgdG8gdGhlIHN1Y2Nlc3Npb24gb2YgYWxnYWwtZG9taW5hbnQgc3BlY2llcywgZXhlcnRpbmcgYSBzaWduaWZpY2FudCBpbmZsdWVuY2Ugb24gdGhlIHJpdmVyIGNhcmJvbiBjeWNsZS4gVGhlIGV4cGFuc2lvbiBvZiBjeWFub2JhY3RlcmlhbCBibG9vbXMgYW5kIHRoZSBjb25zZXF1ZW5jZSBvZiBtZXRhYm9saWMgc3Vic3RyYXRlcyBpbmNyZWFzZSBhcmUgZ3JhZHVhbGx5IGluY3JlYXNpbmcgb3h5Z2VuIGRlcHJpdmF0aW9uLCBldmVuIGh5cG94aWEsIGFuZCBlbmhhbmNpbmcgQ0ggNCBwcm9kdWN0aW9uIGluIHJpdmVycy4gVGhlc2UgZmluZGluZ3Mgc3VnZ2VzdCB0aGF0IHRoZSBzdWNjZXNzaW9uIG9mIGFsZ2FsLWRvbWluYW50IHNwZWNpZXMgY2F1c2VkIGJ5IHJpdmVyIGRhbW1pbmcgaGFzIGEgcHJvbm91bmNlZCBwb3NpdGl2ZSBmZWVkYmFjayBlZmZlY3Qgb24gcmVzZXJ2b2lyIGh5cG94aWEgcHJvY2Vzc2VzIGFuZCBncmVlbmhvdXNlIGVmZmVjdC4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMzM4OS9mZXZvLjIwMjMuMTI5NzA0NyIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cuZnJvbnRpZXJzaW4ub3JnL2FydGljbGVzLzEwLjMzODkvZmV2by4yMDIzLjEyOTcwNDcvcGRmP2lzUHVibGlzaGVkVjI9RmFsc2UiLCJpc3N1ZWREYXRlIjoiMjAyNC0wMS0wNSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZyb250aWVycyBpbiBFY29sb2d5IGFuZCBFdm9sdXRpb24iLCJ2b2x1bWUiOjExLCJpbXBhY3RGYWN0b3IiOiI0Ljc4In0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkZyb250aWVycyBNZWRpYSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NDAsImNpdGVkQnlDb3VudCI6NX19LCJxdW90ZSI6Im9nZW5pYyBudXRyaWVudCBpbnB1dCBzdGltdWxhdGVzIHN1cmZhY2UgZXV0cm9waGljYXRpb24gYW5kIGFsZ2FsIGJsb29tcyBhbmQgdGh1cyBleGFjZXJiYXRlcyBveHlnZW4gZGVwbGV0aW9uIGFuZCBoeXBveGlhIGluIGFxdWF0aWMgZWNvc3lzdGVtcy4gSG93ZXZlciwgZXhpc3Rpbmcgc3R1ZGllcyBoYXZlIHBhaWQgbGl0dGxlIGF0dGVudGlvbiBvbiB0aGUgdW5kZXJzdGFuZGluZyBvZiB0aGUgZmVlZGJhY2sgcmVsYXRpb25zaGlwcyBhbW9uZyBveHlnZW4gY29uc3VtcHRpb24sIGFsZ2FsIHN1Y2Nlc3Npb24gYW5kIGdyZWVuaG91c2UgZ2FzIGVmZmVjdCBhZnRlciByaXZlciBkYW1taW5nLCAuLi4gRE8gcGxheXMgYSBrZXkgcm9sZSBpbiB0aGUgYmlvZ2VvY2hlbWljYWwgY3ljbGUgb2Ygc3VyZmFjZSB3YXRlcnMgYW5kIGlzIGEgc2Vuc2l0aXZlIGluZGljYXRvciBvZiBwaHlzaWNhbCBhbmQgYmlvZ2VvY2hlbWljYWwgY2hhbmdlcyBpbiByZXNlcnZvaXIgYXF1YXRpYyBlY29zeXN0ZW1zICggV2V0emVsLCAyMDAxICkuIFdoZW4gd2F0ZXJzIGVudGVyIGFuIGFuYWVyb2JpYyBzdGF0ZSwgYXF1YXRpYyBvcmdhbmlzbXMgZGllIGluIGxhcmdlIG51bWJlcnMgYW5kIHRoZSByZWxlYXNlIG9mIHNlZGltZW50LXJlZHVjaW5nIHBvbGx1dGFudHMgKGUuZy4sIGFtbW9uaWEgbml0cm9nZW4sIC4uLiAyMDIyXSApLCBmdXJ0aGVyIGRldGVyaW9yYXRpbmcgd2F0ZXIgcXVhbGl0eS4gV2l0aCB0aGUgaW5jcmVhc2Ugb2YgcG9sbHV0aW9uIGxvYWRzIGluIHdhdGVyc2hlZHMgYW5kIHRoZSBpbnRlbnNpZmljYXRpb24gb2Ygd2F0ZXIgZXV0cm9waGljYXRpb24sIHdhdGVyIGh5cG94aWEgaGFzIGJlY29tZSBhIHNldmVyZSBnbG9iYWwgZWNvbG9naWNhbCBhbmQgZW52aXJvbm1lbnRhbCBwcm9ibGVtIGluIHJlc2Vydm9pcnMgKCBTdW4gZXTCoGFsLiwgMjAyMSA7IEphaXN3YWwgZXTCoGFsLiwgMjAyMyApLiBGb2N1c2luZyBvbiB0aGUgZHluYW1pYyBjaGFuZ2VzIGluIn1d. Livestock losses compound this: mass die-offs from neurotoxins in Rewalsar and Chilika depleted draft animals and dairy herds, inflating feed costs and reducing manure availability (Kumar & Mahajan, 2020; Nair & Nayak, 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hdnlhIFZpa3JhbWFuIiwiZmFtaWx5IjoiTmFpciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQcmF0ZWVwIEt1bWFyIiwiZmFtaWx5IjoiTmF5YWsifX1dLCJpZHMiOnsiaXNzbiI6WyIyMjk2LTc3NDUiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzODkvZm1hcnMuMjAyMy4xMDg3Mjk2Iiwib3BlbmFsZXgiOiJXNDM4MjcyMzEzMyJ9LCJ0aXRsZSI6IlVuY292ZXJpbmcgd2F0ZXIgcXVhbGl0eSBhbmQgZXZhbHVhdGluZyB2dWxuZXJhYmlsaXRpZXMgb2Ygc21hbGwtc2NhbGUgZmlzaGVyaWVzIGluIENoaWxpa2EgTGFnb29uLCBJbmRpYSIsImFic3RyYWN0IjoiU21hbGwtc2NhbGUgZmlzaGVyaWVzIChTU0ZzKSBwbGF5IGEgc3Ryb25nIHJvbGUgaW4gc3VzdGFpbmluZyBtaWxsaW9ucyBvZiBsaXZlbGlob29kcywgZm9vZCBzZWN1cml0eSwgbnV0cml0aW9uLCBhbmQgaW5jb21lIGdsb2JhbGx5IGJ1dCB0aGUgZmlzaGVycyBlbmdhZ2VkIGluIHRoaXMgc2VjdG9yIHNpbXVsdGFuZW91c2x5IGV4cGVyaWVuY2UgaGlnaCBsZXZlbHMgb2YgdnVsbmVyYWJpbGl0eSBhbmQgcHJvY2Vzc2VzIG9mIG1hcmdpbmFsaXNhdGlvbi4gU2V2ZXJhbCBmYWN0b3JzIGFyZSBhdHRyaWJ1dGVkIHRvIHRoZSBtdWx0aWRpbWVuc2lvbmFsIHZ1bG5lcmFiaWxpdGllcyB0aGUgc21hbGwtc2NhbGUgZmlzaGVycyBleHBlcmllbmNlLCBzcGFubmluZyBib3RoIG5hdHVyYWwgKGUuZy4sIG5hdHVyYWwgZGlzYXN0ZXJzLCBlY29zeXN0ZW0gY2hhbmdlKSBhbmQgYW50aHJvcG9nZW5pYyAoZS5nLiwgcG9saWN5IGNoYW5nZSwgaHlkcm9sb2dpY2FsIGludGVydmVudGlvbnMsIGFxdWFjdWx0dXJlKSBwcmVzc3VyZXMuIFdoaWxlIHRoZXJlIGlzIG11Y2ggbGl0ZXJhdHVyZSBvbiB2YXJpb3VzIG5hdHVyYWwgYW5kIGh1bWFuIGRyaXZlcnMgb2YgdnVsbmVyYWJpbGl0eSBpbiBzbWFsbC1zY2FsZSBmaXNoZXJ5IGNvbW11bml0aWVzLCBhbiBhYnNlbmNlIG9mIHJlc2VhcmNoIGNvbm5lY3RpbmcgdnVsbmVyYWJpbGl0eSB3aXRoIHdhdGVyIHF1YWxpdHkgaXMgZXZpZGVudC4gRmlzaGVyIGNvbW11bml0aWVzIG9mdGVuIHRhbGsgYWJvdXQgZmlzaCBpbiByZWxhdGlvbiB0byB0aGUgaGVhbHRoIG9mIHRoZWlyIGFxdWF0aWMgaGFiaXRhdHMgd2hlcmVpbiB3YXRlciBxdWFsaXR5IGlzIHNlZW4gYXMgYSBrZXkgcGFyYW1ldGVyLiBUaGUgbGluayBiZXR3ZWVuIGhlYWx0aHkgZmlzaCBhbmQgZ29vZCBxdWFsaXR5IHdhdGVyIGhhcyBzaWduaWZpY2FudCBpbXBsaWNhdGlvbnMgZm9yIHN0cm9uZyBhbmQgdmlhYmxlIGZpc2hpbmcgY29tbXVuaXRpZXMuIFRoaXMgcGFwZXIgZXhhbWluZXMgdGhlc2UgbGlua3MgZnVydGhlciBieSBmb2N1c2luZyBvbiB0aGUgbmF0dXJlIG9mIHZ1bG5lcmFiaWxpdGllcyBjYXVzZWQgYnkgd2F0ZXIgcXVhbGl0eSBjaGFuZ2VzIGluIHRoZSBzbWFsbC1zY2FsZSBmaXNoZXJ5IHN5c3RlbSBvZiBDaGlsaWthIExhZ29vbiBpbiBJbmRpYS4gV2UgdW5kZXJ0YWtlIGRldGFpbGVkIGFuYWx5c2lzIG9mIHRoZSBpbnZhc2l2ZSBzaHJpbXAgYXF1YWN1bHR1cmUgYWN0aXZpdGllcyBhbmQgaHlkcm9sb2dpY2FsIGludGVydmVudGlvbnMgZm9yIG9wZW5pbmcgb2YgYSBsYWdvb24gaW5sZXQgd2l0aCB0aGUgQmF5IG9mIEJlbmdhbCBhcyB0d28gZG9taW5hbnQgZHJpdmVycyBhZHZlcnNlbHkgaW1wYWN0aW5nIHdhdGVyIHF1YWxpdHkgYW5kIGluY3JlYXNpbmcgdnVsbmVyYWJpbGl0aWVzIG9mIHRoZSBlbnRpcmUgc21hbGwtc2NhbGUgZmlzaGVyaWVzIHNvY2lhbC1lY29sb2dpY2FsIHN5c3RlbS4gT3VyIGFuYWx5c2lzIHN1Z2dlc3RzIHRoYXQgdGhlcmUgYXJlIHN0cm9uZyBpbnRlcmNvbm5lY3Rpb25zIGJldHdlZW4gY2hhbmdlcyBpbiB3YXRlciBxdWFsaXR5IGFuZCB0aGUgbGV2ZWxzIG9mIHZ1bG5lcmFiaWxpdGllcyBpbiB0aGUgU1NGcyBvZiBDaGlsaWthIExhZ29vbi4gUG9sbHV0YW50cyBzdWNoIGFzIHBlc3RpY2lkZXMsIGFuZCBvcmdhbmljIGNvbXBvdW5kcyBhY2N1bXVsYXRlIGluIGZpc2ggdGlzc3VlcyBhbmQgYWZmZWN0IHRoZWlyIGdyb3d0aCwgcmVwcm9kdWN0aW9uLCBhbmQgb3ZlcmFsbCBoZWFsdGguIFRoaXMgbGVkIHRvIGRlY2xpbmVzIGluIGZpc2ggcG9wdWxhdGlvbnMsIG1ha2luZyBpdCBtb3JlIGRpZmZpY3VsdCBmb3IgZmlzaGVycyB0byBtYWtlIGEgbGl2aW5nLiBJbiBhZGRpdGlvbiB0byBkaXJlY3QgaW1wYWN0cyBvbiBmaXNoIHBvcHVsYXRpb25zLCBwb29yIHdhdGVyIHF1YWxpdHkgYWxzbyBoYXMgaW5kaXJlY3QgZWZmZWN0cyBvbiB0aGUgc29jaWFsIGFuZCBlY29ub21pYyB2dWxuZXJhYmlsaXR5IG9mIFNTRnMuIEZvciBleGFtcGxlLCBjb250YW1pbmF0aW9uIG9mIHdhdGVyIHNvdXJjZXMgbGVkIHRvIHRoZSByZWR1Y2VkIG51bWJlciBvZiBmaXNoIHNwZWNpZXMgcmVkdWNpbmcgdGhlIGFtb3VudCBvZiB0aW1lIGZpc2hlcnMgY2FuIHNwZW5kIG9uIGZpc2hpbmcgYWN0aXZpdGllcy4gVGhpcyBhbHNvIGFmZmVjdGVkIHRoZSBtYXJrZXRhYmlsaXR5IG9mIGZpc2ggcHJvZHVjdHMsIHJlZHVjZWQgaW5jb21lIGFuZCBpbmNyZWFzZWQgcG92ZXJ0eS4gVG8gZnVsbHkgdW5kZXJzdGFuZCB0aGUgaW50ZXJjb25uZWN0aW9ucyBiZXR3ZWVuIHdhdGVyIHF1YWxpdHkgYW5kIHZ1bG5lcmFiaWxpdHkgaW4gU1NGcyBpbiBDaGlsaWthIExhZ29vbiwgaXQgaXMgaW1wb3J0YW50IHRvIGNvbnNpZGVyIGJvdGggZW52aXJvbm1lbnRhbCBhbmQgc29jaWFsIGZhY3RvcnMsIGFzIHdlbGwgYXMgdGhlIGNvbXBsZXggZmVlZGJhY2sgbG9vcHMgYmV0d2VlbiB0aGVzZSBmYWN0b3JzLiBUaGUgc3R1ZHkgaGVscHMgaW4gYnJpZGdpbmcgYSBjcnVjaWFsIGdhcCBpbiBvdXIgdW5kZXJzdGFuZGluZyBvZiB0aGUgcm9sZSBvZiB3YXRlciBxdWFsaXR5IGluIHZ1bG5lcmFiaWxpdHkgYW5hbHlzaXMgd2l0aGluIHJlc291cmNlIGRlcGVuZGVudCBjb21tdW5pdGllcy4gV2UgY29uY2x1ZGUgd2l0aCBrZXkgaW5zaWdodHMgb24gcG9zc2libGUgY29waW5nIHJlc3BvbnNlcyBhbmQgYWRhcHRpdmUgY2FwYWNpdHkgbmVjZXNzYXJ5IGZvciB0aGUgc21hbGwtc2NhbGUgZmlzaGVyaWVzIGNvbW11bml0aWVzIHRvIHRyYW5zaXRpb24gdG93YXJkIHZpYWJpbGl0eS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMzM4OS9mbWFycy4yMDIzLjEwODcyOTYiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vd3d3LmZyb250aWVyc2luLm9yZy9hcnRpY2xlcy8xMC4zMzg5L2ZtYXJzLjIwMjMuMTA4NzI5Ni9wZGYiLCJpc3N1ZWREYXRlIjoiMjAyMy0wNi0yOSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZyb250aWVycyBpbiBNYXJpbmUgU2NpZW5jZSIsInZvbHVtZSI6MTAsImltcGFjdEZhY3RvciI6IjQuMjgifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRnJvbnRpZXJzIE1lZGlhIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo1MSwiY2l0ZWRCeUNvdW50Ijo1fX0sInF1b3RlIjoiV2F0ZXIgcXVhbGl0eSBvZiBDaGlsaWthIExhZ29vbiBoYXMgZGVncmFkZWQgZHVlIHRvIGFudGhyb3BvZ2VuaWMgaW50ZXJ2ZW50aW9ucyBhbmQgbmF0dXJhbCBmb3JjZXMgb2YgY2hhbmdlLiBWYXJpb3VzIGFjdGl2aXRpZXMgaW4gdGhlIENoaWxpa2EgTGFnb29uLCBzdWNoIGFzIGluZHVzdHJpYWwgd2FzdGV3YXRlciBkaXNwb3NhbCwgc2V3YWdlIGR1bXBpbmcsIGFxdWFjdWx0dXJlLCBoeWRyb2xvZ2ljYWwgaW50ZXJ2ZW50aW9ucywgYW5kIGN5Y2xvbmVzLCBhcmUgcHJvZHVjaW5nIHNhbGluaXR5IGNoYW5nZXMsIHNlZGltZW50IGRlcG9zaXRpb24sIG51dHJpZW50IGVucmljaG1lbnQsIGV1dHJvcGhpY2F0aW9uLCBhbmQgZGVhZCB6b25lcywgYXMgc3RhdGVkLiBGaXNoIHN1cnZpdmFsIGlzIGF0IHJpc2sgYXMgYSByZXN1bHQgb2YgdGhlIHNpZ25pZmljYW50IHJlZHVjdGlvbiBpbiBlbnZpcm9ubWVudGFsIGNpcmN1bXN0YW5jZXMsIHdoaWNoIGRpbWluaXNoZXMgZmlzaGVybWVu4oCZcyBpbmNvbWUgYW5kIHB1c2hlcyB0aGVtIG91dCBvZiBzb2NpZXR5LiBUaGVzZSBjYXVzZXMgYWZmZWN0aW5nIHRoZSB3YXRlciBxdWFsaXR5IG9mIHRoZSBMYWdvb24gZGVwaWN0IG1hbnkgZmFjZXMgb2YgdnVsbmVyYWJpbGl0eSBpbiBTU0YgY29tbXVuaXRpZXMuIEFzIHRoZSBDaGlsaWthIExhZ29vbiBlbnZpcm9ubWVudCBpcyBleHBlY3RlZCB0byB1bmRlcmdvIGluY3JlYXNpbmdseSBkcmFzdGljIGNoYW5nZXMgKHN1Y2ggYXMgY2F0Y2ggY29tcG9zaXRpb24sIGNhdGNoIGNhcGFjaXR5LCBiaW9kaXZlcnNpdHkgdmFyaWF0aW9ucywgd2F0ZXIgcXVhbGl0eSwgYW5kIGZpc2hlcnkgcmV2ZW51ZXMpLCBpbnRlcm5hdGlvbmFsIHBvbGljeSBoYXMgYmVjb21lIG1vcmUgaW50ZXJlc3RlZCBpbiBhZGFwdGF0aW9uIG1lYXN1cmVzLiBHaXZlbiBob3cgZGl2ZXJzZSBTU0YgZWNvc3lzdGVtcyBhcmUgZXhwZWN0ZWQgdG8gYmUgYmV0d2VlbiBjb250aW5lbnRzLCB0aGVyZSBpcyBubyDigJxvbmUgc2l6ZSBmaXRzIGFsbOKAnSBhcHByb2FjaC4gVGhlIHBhcGVyIG91dGxpbmVkIHNldmVyYWwgY29waW5nIGFuZCBhZGFwdGF0aW9uIG1lY2hhbmlzbXMgdG8gaW1wcm92ZSB3YXRlciBxdWFsaXR5IGFuZCBjb250aW51ZSBmaXNoaW5nIGluIGEgc3VzdGFpbmFibGUgd2F5LiBBZGFwdGF0aW9uIHN0cmF0ZWdpZXMgY2FuIG9jY2FzaW9uYWxseSBoYXZlIHVuZm9yZXNlZW4gY29uc2VxdWVuY2VzIGZvciBmaXNoZXJpZXMgYW5kIGZpc2hpbmcgY29tbXVuaXRpZXMuIEFueSB1bmV4cGVjdGVkIGNvbnNlcXVlbmNlcyBvZiBhZGFwdGF0aW9uIGNhbiBiZSBtaXRpZ2F0ZWQgdGhyb3VnaCBsb25nLXRlcm0gcGxhbm5pbmcgYW5kIGlkZW50aWZpY2F0aW9uIG9mIGZ1dHVyZSBvcHRpb25zIHRocm91Z2ggc2NlbmFyaW8gYW5hbHlzaXMuIFRoZSBmaW5kaW5ncyBvZiB0aGlzIHBhcGVyIGNhbiBiZSB1dGlsaXplZCB0byBkZXZlbG9wIHBvbGljaWVzIHRvIHByb3RlY3Qgd2F0ZXIgcXVhbGl0eSBhbmQgU1NGcyBpbiBhIHZhcmlldHkgb2YgY29hc3RhbCBsYWdvb24gc2V0dGluZ3MuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjIyLjI4NTAzNjA4NzAzNjEzMywibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ5MzA1MzQzNjI3OTMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjIzLjc3MzUxOTUxNTk5MTIxLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDgzMzY0MTA1MjI0NjEsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjI1LjI1NTU4NDcxNjc5Njg3NSwibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ4OTU1OTE3MzU4Mzk4NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjYuNzQ0MDY4MTQ1NzUxOTUzLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNTA2ODA1NDE5OTIxODc1LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyOC4yMjYxMzMzNDY1NTc2MTcsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40OTA0NjMyNTY4MzU5NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjkuNzE0NjE2Nzc1NTEyNjk1LCJsZWZ0Ijo4LjMzMTU5MjU1OTgxNDQ1Mywid2lkdGgiOjQwLjQ5NDA5ODY2MzMzMDA4LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozMS4xOTY2ODM4ODM2NjY5OTIsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40OTQzOTIzOTUwMTk1MywiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MzIuNjg1MTY1NDA1MjczNDQsImxlZnQiOjguMzMxNTk2Mzc0NTExNzE5LCJ3aWR0aCI6NDAuNDg3NDQ1ODMxMjk4ODMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM0LjE2NzIzMjUxMzQyNzczNCwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40OTc3NDU1MTM5MTYwMTYsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM1LjY1NTcxNTk0MjM4MjgxLCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ5ODYzODE1MzA3NjE3LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozNy4xMzc3ODMwNTA1MzcxMSwibGVmdCI6MTAuNjg1MTg2Mzg2MTA4Mzk4LCJ3aWR0aCI6MzguMTQxMTA1NjUxODU1NDcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM4LjYyNjI2NjQ3OTQ5MjE5LCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4NzE1MjA5OTYwOTM3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDAuMTA4MzMzNTg3NjQ2NDg0LCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4OTg1NjcxOTk3MDcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQxLjU5NjgxMzIwMTkwNDMsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDg4NjY2NTM0NDIzODMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQzLjA3ODg4MDMxMDA1ODU5NCwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40ODE4NjExMTQ1MDE5NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDQuNTY3MzYzNzM5MDEzNjcsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDgzMDU4OTI5NDQzMzYsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQ2LjA0OTQzMDg0NzE2Nzk3LCJsZWZ0Ijo4LjMzMzEwODkwMTk3NzUzOSwid2lkdGgiOjQwLjUwMDc0NzY4MDY2NDA2LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo0Ny41Mzc5MTQyNzYxMjMwNSwibGVmdCI6OC4zMzMxMDg5MDE5Nzc1MzksIndpZHRoIjo0MC40OTU2MDU0Njg3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDkuMDE5OTgxMzg0Mjc3MzQ0LCJsZWZ0Ijo4LjMzMzEwNjA0MDk1NDU5LCJ3aWR0aCI6NDAuNTA2ODE2ODY0MDEzNjcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUwLjUwODQ2NDgxMzIzMjQyLCJsZWZ0Ijo4LjMzMzEwNjA0MDk1NDU5LCJ3aWR0aCI6NDAuNDg4NTA2MzE3MTM4NjcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUxLjk5MDUyODEwNjY4OTQ1LCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDk3NzI2NDQwNDI5NjksImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUzLjQ3OTAxMTUzNTY0NDUzLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6MzMuMTU5NDQ2NzE2MzA4NTk0LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo1NC45NjEwNzg2NDM3OTg4MywibGVmdCI6MTAuNjgzNjcwMDQzOTQ1MzEyLCJ3aWR0aCI6MzguMTM2NTk2Njc5Njg3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NTYuNDQ5NTYyMDcyNzUzOTA2LCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDg3MTQ0NDcwMjE0ODQ0LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEzIn19LHsib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlBhd2FuIiwiZmFtaWx5IjoiS3VtYXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW1icmlzaCBLdW1hciIsImZhbWlseSI6Ik1haGFqYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyNTIzLTM5NjMiLCIyNTIzLTM5NzEiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMDcvczQyNDUyLTAyMC0zMDgyLTgiLCJtYWciOiIzMDM1OTYwMjI4Iiwib3BlbmFsZXgiOiJXMzAzNTk2MDIyOCJ9LCJ0aXRsZSI6IlRyb3BoaWMgc3RhdHVzIGFuZCBpdHMgcmVndWxhdGluZyBmYWN0b3IgZGV0ZXJtaW5hdGlvbiBhdCB0aGUgUmV3YWxzYXIgTGFrZSwgbm9ydGh3ZXN0IEhpbWFsYXlhIChIUCksIEluZGlhLCBiYXNlZCBvbiBzZWxlY3RlZCBwYXJhbWV0ZXJzIGFuZCBtdWx0aXZhcmlhdGUgc3RhdGlzdGljYWwgYW5hbHlzaXMiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAwNy9zNDI0NTItMDIwLTMwODItOCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9saW5rLnNwcmluZ2VyLmNvbS9jb250ZW50L3BkZi8xMC4xMDA3L3M0MjQ1Mi0wMjAtMzA4Mi04LnBkZiIsImlzc3VlZERhdGUiOiIyMDIwLTA2LTIyIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiU04gQXBwbGllZCBTY2llbmNlcyIsImlzc3VlIjo3LCJ2b2x1bWUiOjIsImltcGFjdEZhY3RvciI6IjcuMzUifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiU3ByaW5nZXIgTmF0dXJlIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo0MCwiY2l0ZWRCeUNvdW50IjoxOX19LCJxdW90ZSI6IkNoYW5naW5nIHF1YWxpdHkgaGFzIGNyZWF0ZWQgdGhlIHdhdGVyIHNjYXJjaXR5IGFuZCAgYSBodWdlIG1hc3Mgb2YgcG9wdWxhdGlvbiB0aHVzIGJlZW4gZGVwcml2ZWQgZnJvbSB1c2luZyAgdGhlIGZyZXNod2F0ZXIgZm9yIHZhcmlvdXMgcHVycG9zZXMgWzMsIDRdLiBIb3dldmVyLCB0aGUgIG9ibm94aW91cyBtYXRlcmlhbHMgdGhhdCBzdGF5IGluIHdhdGVyIGJvZGllcyBmb3IgYSBsb25nLXRpbWUgcGVyaW9kIGJyaW5nIGNoYW5nZXMgaW4gdGhlIHBoeXNpb2NoZW1pY2FsIGNoYXItYWN0ZXJpc3RpY3MgYXMgd2VsbCBhcyB0aGUgbmF0dXJhbCBzY2VuYXJpbyBvZiB0aGUgbGFrZXMgWzVdLiAgVGhlIEhpbWFsYXlhbiBsYWtlcyBhcmUgdW5kZXIgc3RyZXNzIGR1ZSB0byBtaXNtYW5hZ2VkIGh1bWFuIGFjdGl2aXRpZXMsIHdoaWNoIGluY3JlYXNlIG51dHJpZW50cyBhbmQgcG9sbHV0aW9uIGluIHRlcm1zIG9mIGhlYXZ5IG1ldGFscyB0b3hpY2l0eSBhbmQgYmFjdGVyaWFsIGNvbnRhbWluYXRpb24gIFs24oCTOF0uIFRoZSBtYW5hZ2VtZW50IGFuZCBwcmVzZXJ2YXRpb24gb2YgdGhlIGxha2VzIGF0ICBwcmVzZW50IGhhdmUgYmVjb21lIGEgZ3JlYXQgY2hhbGxlbmdlIGZvciBlbnZpcm9ubWVudGFsLWlzdHMgYW5kIG1hbmFnZXJzIFs1LCA5XS4gVGhlIGV1dHJvcGhpY2F0aW9uIGlzIGFsc28gYSBtYWluIHByb2JsZW0gb2YgdGhlICBmcmVzaHdhdGVyIGxha2VzLCBhbmQgaXQgaXMgc3BlY2lmaWVkIGZvciB0aGUgbGFrZXMgaGF2aW5nICBudXRyaWVudCBlbnJpY2htZW50LiBJdCBjYXVzZXMgZXhjZXNzaXZlIGdyb3d0aCBvZiBhbGdhZSAgYW5kIGlzIGNvbnNpZGVyZWQgdG8gYmUgcmVzcG9uc2libGUgZm9yIGRlZ3JhZGluZyB0aGUgIHF1YWxpdGF0aXZlIG5hdHVyZSBvZiB0aGUgbGFrZXMgWzUsIDEwXS4gVGhlIGV1dHJvcGhpY2F0aW9uICBpcyBmb2xsb3dlZCBieSB0aGUgYWxnYWwgYmxvb20sIHdoaWNoIGdlbmVyYXRlcyB0b3hpbiwgIHdvcnNlbnMgd2F0ZXIgcXVhbGl0eSwgZGVwbGV0ZXMgZGlzc29sdmVkIG94eWdlbiAoRE8pICogUGF3YW4gS3VtYXIsIHBhd2FuLmV2c0BnbWFpbC5jb20gfCAxRGVwYXJ0bWVudCBvZiBFbnZpcm9ubWVudGFsIFNjaWVuY2VzLCBDZW50cmFsIFVuaXZlcnNpdHkgb2YgSGltYWNoYWwgUHJhZGVzaCwgRGhhcmFtc2hhbGEgMTc2MjE1LCBJbmRpYS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2OC41Nzg0NjA2OTMzNTkzOCwibGVmdCI6OC41NzE0MTg3NjIyMDcwMzEsIndpZHRoIjo0MC4zMjc1MTQ2NDg0Mzc1LCJoZWlnaHQiOjEuNTE5ODU1MjYwODQ4OTk5fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NzAuMTU4OTk2NTgyMDMxMjUsImxlZnQiOjguNTY5NzM4Mzg4MDYxNTIzLCJ3aWR0aCI6NDAuMzU0NzI0ODg0MDMzMiwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjcxLjczOTU0NzcyOTQ5MjE5LCJsZWZ0Ijo4LjU2OTczODM4ODA2MTUyMywid2lkdGgiOjQwLjM0OTY5MzI5ODMzOTg0NCwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjczLjMyMDA5MTI0NzU1ODYsImxlZnQiOjguNTY4MDU5OTIxMjY0NjQ4LCJ3aWR0aCI6NDAuMDIwMDg4MTk1ODAwNzgsImhlaWdodCI6MS41MTk4NTUyNjA4NDg5OTl9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo3NC45MDA2NDIzOTUwMTk1MywibGVmdCI6OC41NjgwNTk5MjEyNjQ2NDgsIndpZHRoIjozOS45OTk5Mjc1MjA3NTE5NSwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjc2LjQ4MTE4NTkxMzA4NTk0LCJsZWZ0Ijo4LjU2ODA1NzA2MDI0MTcsIndpZHRoIjo0MC4zNTcwNzQ3Mzc1NDg4MywiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjc4LjA2MTcyOTQzMTE1MjM0LCJsZWZ0Ijo4LjU2OTczNjQ4MDcxMjg5LCJ3aWR0aCI6NDAuMzUzNzEzOTg5MjU3ODEsImhlaWdodCI6MS41MTk4NTUyNjA4NDg5OTl9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2MC42NzU3Mjc4NDQyMzgyOCwibGVmdCI6NTEuNDI3MTY1OTg1MTA3NDIsIndpZHRoIjo0MC4zNzg1NjY3NDE5NDMzNiwiaGVpZ2h0IjoxOC45MDU4NTUxNzg4MzMwMDh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo4MS45MjczMzc2NDY0ODQzOCwibGVmdCI6OC40MDE0OTc4NDA4ODEzNDgsIndpZHRoIjo3My45NTQ2OTY2NTUyNzM0NCwiaGVpZ2h0IjoyLjY3ODc2MjY3NDMzMTY2NX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEifX1d. Smallholder farmers, reliant on lake-irrigated fields, report net losses of INR 10,000-25,000 per hectare annually, fueling debt cycles and land abandonment (Paerl et al., 2019)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhhbnMgVy4iLCJmYW1pbHkiOiJQYWVybCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLYXJsIEUuIiwiZmFtaWx5IjoiSGF2ZW5zIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hdGhhbiBTLiIsImZhbWlseSI6IkhhbGwifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiVGltb3RoeSBHLiIsImZhbWlseSI6Ik90dGVuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1lbmd5dWFuIiwiZmFtaWx5IjoiWmh1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhhaSIsImZhbWlseSI6Ilh1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikd1YW5nd2VpIiwiZmFtaWx5IjoiWmh1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkJvcWlhbmciLCJmYW1pbHkiOiJRaW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIxMzIzLTE2NTAiLCIxNDQ4LTYwNTkiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwNzEvbWYxODM5MiIsIm1hZyI6IjI5NDIzMjE1NDIiLCJvcGVuYWxleCI6IlcyOTQyMzIxNTQyIn0sInRpdGxlIjoiTWl0aWdhdGluZyBhIGdsb2JhbCBleHBhbnNpb24gb2YgdG94aWMgY3lhbm9iYWN0ZXJpYWwgYmxvb21zOiBjb25mb3VuZGluZyBlZmZlY3RzIGFuZCBjaGFsbGVuZ2VzIHBvc2VkIGJ5IGNsaW1hdGUgY2hhbmdlIiwiYWJzdHJhY3QiOiJNYW5hZ2luZyBhbmQgbWl0aWdhdGluZyB0aGUgZ2xvYmFsIGV4cGFuc2lvbiBvZiB0b3hpYyBjeWFub2JhY3RlcmlhbCBoYXJtZnVsIGFsZ2FsIGJsb29tcyAoQ3lhbm9IQUJzKSBpcyBhIG1ham9yIGNoYWxsZW5nZSBmYWNpbmcgcmVzZWFyY2hlcnMgYW5kIHdhdGVyIHJlc291cmNlIG1hbmFnZXJzLiBWYXJpb3VzIGFwcHJvYWNoZXMsIGluY2x1ZGluZyBudXRyaWVudCBsb2FkIHJlZHVjdGlvbiwgYXJ0aWZpY2lhbCBtaXhpbmcgYW5kIGZsdXNoaW5nLCBvbW5pdm9yb3VzIGZpc2ggcmVtb3ZhbCwgYWxnYWVjaWRlIGFwcGxpY2F0aW9ucyBhbmQgc2VkaW1lbnQgZHJlZGdpbmcsIGhhdmUgYmVlbiB1c2VkIHRvIHJlZHVjZSBibG9vbSBvY2N1cnJlbmNlcy4gSG93ZXZlciwgbWFuYWdlcnMgbm93IGZhY2UgdGhlIGFkZGl0aW9uYWwgY2hhbGxlbmdlIG9mIGhhdmluZyB0byBhZGRyZXNzIHRoZSBlZmZlY3RzIG9mIGNsaW1hdGUgY2hhbmdlIG9uIHdhdGVyc2hlZCBoeWRyb2xvZ2ljYWwgYW5kIG51dHJpZW50IGxvYWQgZHluYW1pY3MsIHdhdGVyIHRlbXBlcmF0dXJlLCBtaXhpbmcgcmVnaW1lIGFuZCBpbnRlcm5hbCBudXRyaWVudCBjeWNsaW5nLiBSaXNpbmcgdGVtcGVyYXR1cmVzIGFuZCBpbmNyZWFzaW5nIGZyZXF1ZW5jaWVzIGFuZCBtYWduaXR1ZGVzIG9mIGV4dHJlbWUgd2VhdGhlciBldmVudHMsIGluY2x1ZGluZyB0cm9waWNhbCBjeWNsb25lcywgZXh0cmF0cm9waWNhbCBzdG9ybXMsIGZsb29kcyBhbmQgZHJvdWdodHMsIGFsbCBwcm9tb3RlIEN5YW5vSEFCcyBhbmQgYWZmZWN0IHRoZSBlZmZpY2FjeSBvZiBlY29zeXN0ZW0gcmVtZWRpYXRpb24gbWVhc3VyZXMuIFRoZXNlIGNsaW1hdGljIGNoYW5nZXMgd2lsbCBsaWtlbHkgcmVxdWlyZSBzZXR0aW5nIHN0cmljdGVyIG51dHJpZW50IChpbmNsdWRpbmcgYm90aCBuaXRyb2dlbiBhbmQgcGhvc3Bob3J1cykgcmVkdWN0aW9uIHRhcmdldHMgZm9yIGJsb29tIGNvbnRyb2wgaW4gYWZmZWN0ZWQgd2F0ZXJzLiBJbiBhZGRpdGlvbiwgdGhlIGVmZmljYWN5IG9mIGN1cnJlbnRseSB1c2VkIG1ldGhvZHMgdG8gcmVkdWNlIEN5YW5vSEFCcyB3aWxsIG5lZWQgdG8gYmUgcmUtZXZhbHVhdGVkIGluIGxpZ2h0IG9mIHRoZSBzeW5lcmdpc3RpYyBlZmZlY3RzIG9mIGNsaW1hdGUgY2hhbmdlIHdpdGggbnV0cmllbnQgZW5yaWNobWVudC4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTA3MS9tZjE4MzkyIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2Nkci5saWIudW5jLmVkdS9kb3dubG9hZHMvMDU3NDIyODM0IiwiaXNzdWVkRGF0ZSI6IjIwMTktMDMtMjUiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTEwIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJNYXJpbmUgYW5kIEZyZXNod2F0ZXIgUmVzZWFyY2giLCJpc3N1ZSI6NSwidm9sdW1lIjo3MSwiaW1wYWN0RmFjdG9yIjoiMi43MCJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjU3OSwibGFzdFBhZ2UiOjU3OX19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJDU0lSTyBQdWJsaXNoaW5nIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjoxMDQsImNpdGVkQnlDb3VudCI6OTF9fSwicXVvdGUiOiJUaGF0IHJlbGF0aW9uc2hpcCBoYXMgc2VydmVkIGFzIHRoZSBmb3VuZGF0aW9uIGZvciBzZXR0aW5nICBudXRyaWVudCBsb2FkaW5nIHRhcmdldHMgaW4gbGFrZXMgYXJvdW5kIHRoZSB3b3JsZCwgYW5kIGNhcmVmdWxseSAgZGVzaWduZWQgZXhwZXJpbWVudHMgbWF5IGVsdWNpZGF0ZSB0byB3aGF0IGV4dGVudCB0ZW1wZXJhdHVyZSAgaW5jcmVhc2UgYWZmZWN0cyB0aGUgdGhyZXNob2xkIGF0IHdoaWNoIHRoZSBmcmVxdWVuY3kgb2Ygb2NjdXIgIHJlbmNlIG9mIEN5YW5vSEFCcyBxdWlja2x5IHJpc2VzIHdpdGggaW5jcmVhc2luZyBOIGFuZCBQLiBDb25jbHVzaW9ucyBSZWNlbnQgc3R1ZGllcyBpbmRpY2F0ZSB0aGF0IHRoZSBzeW5lcmdpc3RpYyBjb21iaW5hdGlvbiBvZiBhbnRocm9wb2dlbmljIG51dHJpZW50IGxvYWRpbmcsIHJpc2luZyB0ZW1wZXJhdHVyZXMsIGVuaGFuY2VkICB2ZXJ0aWNhbCBzdHJhdGlmaWNhdGlvbiwgaW5jcmVhc2VkIHJlc2lkZW5jZSB0aW1lIGFuZCBtb3JlICBleHRyZW1lIGNsaW1hdGljIGNvbmRpdGlvbnMgb3ZlcmFsbCB3aWxsIGZhdm91ciBjeWFub2JhY3RlcmlhbCAgZG9taW5hbmNlIGluIGEgd2lkZSByYW5nZSBvZiBhcXVhdGljIGVjb3N5c3RlbXMuIFRoZSBleHBhbiAgc2lvbiBvZiBDeWFub0hBQnMgaGFzIHNlcmlvdXMgY29uc2VxdWVuY2VzIGZvciBodW1hbiB3YXRlciAgc3VwcGxpZXMsIGZpc2hlcmllcyBhbmQgcmVjcmVhdGlvbmFsIHJlc291cmNlcy4gRWNvc3lzdGVtIHNjYWxlICBwaHlzaWNhbCBjaGVtaWNhbCBiaW90aWMgbWl0aWdhdGlvbiBzdHJhdGVnaWVzIHdpbGwgaGF2ZSB0byAgaW5jb3Jwb3JhdGUgbnV0cmllbnQgKG1vc3Qgb2Z0ZW4gYm90aCBOIGFuZCBQKSBpbnB1dCByZWR1Y3Rpb25zLiAgVGhlIG1hZ25pdHVkZXMgb2YgdGhlc2UgcmVkdWN0aW9ucyB3aWxsIG5lZWQgdG8gYmUgc3lzdGVtICBzcGVjaWZpYyBhbmQgd2lsbCBsaWtlbHkgbmVlZCB0byBiZSBhZGp1c3RlZCB3aXRoIGNoYW5naW5nIGNsaSAgbWF0aWMgY29uZGl0aW9ucy4gU3BlY2lmaWNhbGx5LCB3YXRlciBxdWFsaXR5IG1hbmFnZXJzIHdpbGwgaGF2ZSAgdG8gYWNjb21tb2RhdGUgd2FybWluZyBhbmQgZ3JlYXRlciBoeWRyb2xvZ2ljYWwgdmFyaWFiaWxpdHkgIGFuZCBleHRyZW1lbmVzcyByZXN1bHRpbmcgZnJvbSBjbGltYXRlIGNoYW5nZSBpbiBmb3JtdWxhdGluZyAgdGhlaXIgc3RyYXRlZ2llcy4gQSBrZXkgbG9uZyB0ZXJtIGNvbnRyb2wgd2UgY2FuIGV4ZXJ0IHRvIHJlZHVjZSAgdGhlIHJhdGUgYW5kIGV4dGVudCBvZiBnbG9iYWwgd2FybWluZyBpcyBjdXJiaW5nIGdyZWVuaG91c2UgZ2FzICBlbWlzc2lvbnMuIFdpdGhvdXQgdGhpcyBlc3NlbnRpYWwgc3RlcCwgZnV0dXJlIHdhcm1pbmcgdHJlbmRzLCAgaHlkcm9sb2dpY2FsIGV4dHJlbWVzIChyZWNvcmQgcmFpbmZhbGxzIGFuZCBkcm91Z2h0cykgYW5kIHRoZWlyICBlZmZlY3RzIG9uIGFxdWF0aWMgZWNvc3lzdGVtcyB3aWxsIGNvbnRpbnVlIHRvIHByb21vdGUgb3Bwb3IgIHR1bmlzdGljIEN5YW5vSEFCcy4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6MTQuMTQ5MjMzODE4MDU0MiwibGVmdCI6MTMuMTI4NTk1MzUyMTcyODUyLCJ3aWR0aCI6MzcuMDgyMTcyMzkzNzk4ODMsImhlaWdodCI6MS41MTY2MDIyNzc3NTU3MzczfX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjE1LjQ1MTUwODUyMjAzMzY5MSwibGVmdCI6MTMuMTI4NTQ0ODA3NDM0MDgyLCJ3aWR0aCI6MzcuMDg0ODMxMjM3NzkyOTcsImhlaWdodCI6MS41MTY2MDMyMzE0MzAwNTM3fX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjE2Ljc1Mzc4NDE3OTY4NzUsImxlZnQiOjEzLjEyODU0MjkwMDA4NTQ1LCJ3aWR0aCI6MzcuMDgzMjA5OTkxNDU1MDgsImhlaWdodCI6MS41MTY2MDQxODUxMDQzNzAxfX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjE4LjA1NjA1MzE2MTYyMTA5NCwibGVmdCI6MTMuMTI4NTQ0ODA3NDM0MDgyLCJ3aWR0aCI6MzYuNjE1MDc0MTU3NzE0ODQ0LCJoZWlnaHQiOjEuNTE2NjA0MTg1MTA0MzcwMX19LHsicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjoxOS40NjgzNzgwNjcwMTY2LCJsZWZ0IjoxMy4xMjg1NDQ4MDc0MzQwODIsIndpZHRoIjozNC4yOTA1MzExNTg0NDcyNjYsImhlaWdodCI6MS4yNDUwMTEyMTA0NDE1ODk0fX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjIyLjIyNjE0Mjg4MzMwMDc4LCJsZWZ0IjoxMy4xMjg1NTA1Mjk0Nzk5OCwid2lkdGgiOjcuNjQ5ODczNzMzNTIwNTA4LCJoZWlnaHQiOjEuMTM1NDgzMTQ1NzEzODA2Mn19LHsicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjoyMy43MzQ4NDk5Mjk4MDk1NywibGVmdCI6MTMuMTI4NTM1MjcwNjkwOTE4LCJ3aWR0aCI6MzcuMDg5Mjg2ODA0MTk5MjIsImhlaWdodCI6MjYuMDg4NjQwMjEzMDEyNjk1fX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNTg5In19XQ%3D%3D. Climate feedbacks prolonged stratification and warming project 2-3 fold bloom intensification by 2050, threatening food security for 100 million lake-dependent agrarians (Jansen et al., 2024; Nõges et al., 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkpvYWNoaW0iLCJmYW1pbHkiOiJKYW5zZW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiR2F2aW4gTC4iLCJmYW1pbHkiOiJTaW1wc29uIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikdlc2EgQS4iLCJmYW1pbHkiOiJXZXloZW5tZXllciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJMYXVyYSBILiIsImZhbWlseSI6IkjDpHJrw7ZuZW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW5kcmV3IE0uIiwiZmFtaWx5IjoiUGF0ZXJzb24ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUGF1bCBBLiBkZWwiLCJmYW1pbHkiOiJHaW9yZ2lvIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ill2ZXMgVC4iLCJmYW1pbHkiOiJQcmFpcmllIn19XSwiaWRzIjp7Imlzc24iOlsiMTc1OC02NzhYIiwiMTc1OC02Nzk4Il0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDM4L3M0MTU1OC0wMjQtMDIwNTgtMyIsIm9wZW5hbGV4IjoiVzQ0MDAyMDgyMDAiLCJleGEiOiJodHRwczovL3d3dy5uYXR1cmUuY29tL2FydGljbGVzL3M0MTU1OC0wMjQtMDIwNTgtMz9lcnJvcj1jb29raWVzX25vdF9zdXBwb3J0ZWQmY29kZT1hNzRhMjVmZi1hNThkLTRkZjgtOTM2Zi0yNjJlMmUzZTkxOTgifSwidGl0bGUiOiJDbGltYXRlLWRyaXZlbiBkZW94eWdlbmF0aW9uIG9mIG5vcnRoZXJuIGxha2VzIiwiYWJzdHJhY3QiOiJBYnN0cmFjdCBPeHlnZW4gZGVwbGV0aW9uIGNvbnN0aXR1dGVzIGEgbWFqb3IgdGhyZWF0IHRvIGxha2UgZWNvc3lzdGVtcyBhbmQgdGhlIHNlcnZpY2VzIHRoZXkgcHJvdmlkZS4gTW9zdCBvZiB0aGUgd29ybGTigJlzIGxha2VzIGFyZSBsb2NhdGVkICZndDs0NcKwIE4sIHdoZXJlIGFjY2VsZXJhdGVkIGNsaW1hdGUgd2FybWluZyBhbmQgZWxldmF0ZWQgY2FyYm9uIGxvYWRzIG1pZ2h0IHNldmVyZWx5IGluY3JlYXNlIHRoZSByaXNrIG9mIGh5cG94aWEsIGJ1dCB0aGlzIGhhcyBub3QgYmVlbiBzeXN0ZW1hdGljYWxseSBleGFtaW5lZC4gSGVyZSBhbmFseXNpcyBvZiAyLjYgbWlsbGlvbiB3YXRlciBxdWFsaXR5IG9ic2VydmF0aW9ucyBmcm9tIDgsMjg4IGxha2VzIHNob3dzIHRoYXQgYmV0d2VlbiAxOTYwIGFuZCAyMDIyLCBtb3N0IG5vcnRoZXJuIGxha2VzIGV4cGVyaWVuY2VkIHJhcGlkIGRlb3h5Z2VuYXRpb24gc3Ryb25nbHkgbGlua2VkIHRvIGNsaW1hdGUtZHJpdmVuIHByb2xvbmdhdGlvbiBvZiBzdW1tZXIgc3RyYXRpZmljYXRpb24uIE94eWdlbiBsZXZlbHMgZGV0ZXJpb3JhdGVkIG1vc3QgaW4gc21hbGwgbGFrZXMgKCZsdDsxMCBoYSkgb3dpbmcgdG8gdGhlaXIgZ3JlYXRlciB2b2x1bWV0cmljIG94eWdlbiBkZW1hbmQgYW5kIHN1cmZhY2Ugd2FybWluZyByYXRlcywgd2hpbGUgdGhlIGxhcmdlc3QgbGFrZXMgZ2FpbmVkIG94eWdlbiB1bmRlciBtaW5pbWFsIHN0cmF0aWZpY2F0aW9uIGNoYW5nZXMgYW5kIGltcHJvdmVkIGFlcmF0aW9uIGF0IHNwcmluZyBvdmVydHVybnMuIFNlYXNvbmFsIG94eWdlbiBjb25zdW1wdGlvbiByYXRlcyBkZWNsaW5lZCwgZGVzcGl0ZSB3aWRlc3ByZWFkIGJyb3duaW5nLiBQcm9saWZlcmF0aW5nIGFub3hpYSBlbmhhbmNlZCBzZWFzb25hbCBpbnRlcm5hbCBsb2FkaW5nIG9mIEMsIFAgYW5kIE4gYnV0IGRlcGxldGVkIFAgbG9uZy10ZXJtLCBpbmRpY2F0aW5nIHRoYXQgZGVveHlnZW5hdGlvbiBjYW4gZXhoYXVzdCByZWRveC1zZW5zaXRpdmUgZnJhY3Rpb25zIG9mIHNlZGltZW50IG51dHJpZW50IHJlc2Vydm9pcnMuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjEwMzgvczQxNTU4LTAyNC0wMjA1OC0zIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAzOC9zNDE1NTgtMDI0LTAyMDU4LTMiLCJpc3N1ZWREYXRlIjoiMjAyNC0wNy0wMSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTEiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6Ik5hdHVyZSBDbGltYXRlIENoYW5nZSIsImlzc3VlIjo4LCJ2b2x1bWUiOjE0LCJpbXBhY3RGYWN0b3IiOiIxNS4wMiJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjgzMiwibGFzdFBhZ2UiOjgzOH19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJOYXR1cmUgUG9ydGZvbGlvIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo4MCwiY2l0ZWRCeUNvdW50IjozN319LCJxdW90ZSI6IkFlcm9iaWMgcmVzcGlyYXRpb24gY2FuIGRlcGxldGUgb3h5Z2VuIGNvbmNlbnRyYXRpb25zIGF0IGRlcHRoIHdoZW4gc2Vhc29uYWwgc3RyYXRpZmljYXRpb24gcHJldmVudHMgYWVyYXRpb24gb2YgdGhlIHdhdGVyIGNvbHVtbls1XShodHRwczovL3d3dy5uYXR1cmUuY29tL2FydGljbGVzL3M0MTU1OC0wMjQtMDIwNTgtMyNyZWYtQ1I1KS4gVGhpcyBkZXRyaW1lbnRhbCBjb25kaXRpb24sIHRlcm1lZCBoeXBveGlhIChET%2BKAiTzigIky4oCJbWfigIls4oiSMSksIGRpbWluaXNoZXMgdGhlIGNvbGQtd2F0ZXIgaGFiaXRhdCBvZiBhcXVhdGljIGxpZmUsIGluY2x1ZGluZyBjb21tZXJjaWFsbHkgaW1wb3J0YW50IGZpc2hbNl0oaHR0cHM6Ly93d3cubmF0dXJlLmNvbS9hcnRpY2xlcy9zNDE1NTgtMDI0LTAyMDU4LTMjcmVmLUNSNikgYW5kIGNhbiBjYXVzZSBtYXNzIG1vcnRhbGl0eSBpZiBwZXJzaXN0ZW50WzddKGh0dHBzOi8vd3d3Lm5hdHVyZS5jb20vYXJ0aWNsZXMvczQxNTU4LTAyNC0wMjA1OC0zI3JlZi1DUjcpLiBBbm94aWEgKERP4oCJPOKAiTAuNeKAiW1n4oCJbOKIkjEpIHByb21vdGVzIHRoZSByZWR1Y3RpdmUgZGlzc29sdXRpb24gb2YgbWluZXJhbC1ib3VuZCBjYXJib24gYW5kIHBob3NwaG9ydXNbOF0oaHR0cHM6Ly93d3cubmF0dXJlLmNvbS9hcnRpY2xlcy9zNDE1NTgtMDI0LTAyMDU4LTMjcmVmLUNSOCksIFs5XShodHRwczovL3d3dy5uYXR1cmUuY29tL2FydGljbGVzL3M0MTU1OC0wMjQtMDIwNTgtMyNyZWYtQ1I5KSB3aGljaCBmdWVscyBoYXJtZnVsIGFsZ2FsIGJsb29tc1sxMF0oaHR0cHM6Ly93d3cubmF0dXJlLmNvbS9hcnRpY2xlcy9zNDE1NTgtMDI0LTAyMDU4LTMjcmVmLUNSMTApIGFuZCBsaW1pdHMgbWljcm9iaWFsIG94aWRhdGlvbiBvZiBtZXRoYW5lLCBhIHBvdGVudCBncmVlbmhvdXNlIGdhc1sxMV0oaHR0cHM6Ly93d3cubmF0dXJlLmNvbS9hcnRpY2xlcy9zNDE1NTgtMDI0LTAyMDU4LTMjcmVmLUNSMTEpLiJ9LHsib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRpaW5hIiwiZmFtaWx5IjoiTsO1Z2VzIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hc2ltZSIsImZhbWlseSI6IkphbmF0aWFuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlJlZXQiLCJmYW1pbHkiOiJMYXVnYXN0ZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQZWV0ZXIiLCJmYW1pbHkiOiJOw7VnZXMifX1dLCJpZHMiOnsiaXNzbiI6WyIyMzUxLTk4OTQiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMTYvai5nZWNjby4yMDIwLmUwMTM2OSIsIm1hZyI6IjMxMDU0NDYwOTAiLCJvcGVuYWxleCI6IlczMTA1NDQ2MDkwIiwiZXhhIjoiaHR0cHM6Ly93d3cuc2NpZW5jZWRpcmVjdC5jb20vc2NpZW5jZS9hcnRpY2xlL3BpaS9TMjM1MTk4OTQyMDMwOTEwMCJ9LCJ0aXRsZSI6IlBvc3Qtc292aWV0IGNoYW5nZXMgaW4gbml0cm9nZW4gYW5kIHBob3NwaG9ydXMgc3RvaWNoaW9tZXRyeSBpbiB0d28gbGFyZ2Ugbm9uLXN0cmF0aWZpZWQgbGFrZXMgYW5kIHRoZSBpbXBhY3Qgb24gcGh5dG9wbGFua3RvbiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2ouZ2VjY28uMjAyMC5lMDEzNjkiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2ouZ2VjY28uMjAyMC5lMDEzNjkiLCJpc3N1ZWREYXRlIjoiMjAyMC0xMS0yMCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTEiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6Ikdsb2JhbCBFY29sb2d5IGFuZCBDb25zZXJ2YXRpb24iLCJ2b2x1bWUiOjI0LCJpbXBhY3RGYWN0b3IiOiIzLjU5In0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkVsc2V2aWVyIEJWIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo3MSwiY2l0ZWRCeUNvdW50IjoxNH19LCJxdW90ZSI6IlJhbmRvbSBGb3Jlc3QgYW5hbHlzaXMgaW5kaWNhdGVkIGFuIGltcG9ydGFudCByb2xlIG9mIGxpZ2h0IGxpbWl0YXRpb24gaW4gYm90aCBsYWtlcy4gQ29uc3RhbnRseSBsb3dlciBsZXZlbHMgb2YgTiBpbiB0aGUgZGVlcGVyIExha2UgUGVpcHNpIGZhdm91cmVkIE4yLWZpeGluZyBzcGVjaWVzLCB3aGljaCwgYXMgYSBwYXJhZG94LCBiZWNhbWUgUC1saW1pdGVkLiBBcyBjbGltYXRlIHdhcm1pbmcgcmVpbmZvcmNlcyBldXRyb3BoaWNhdGlvbiBwaGVub21lbmEgaW4gbGFrZXMgYnkgaW5jcmVhc2luZyBpbnRlcm5hbCBudXRyaWVudCBsb2FkaW5nIGFuZCBmYXZvdXJpbmcgYmxvb20tZm9ybWluZyBjeWFub2JhY3RlcmlhLCBtb3JlIHN0cmluZ2VudCBtZWFzdXJlcyB3b3VsZCBiZSBuZWVkZWQgdG8gZnVydGhlciBsaW1pdCBudXRyaWVudCBsb2FkcyAoZXNwZWNpYWxseSB0aGF0IG9mIFApIHRvIGxha2VzIHRocm91Z2ggaW1wcm92ZWQgd2FzdGV3YXRlciB0cmVhdG1lbnQgYW5kIGluY3JlYXNlZCBlZmZpY2llbmN5IG9mIGZlcnRpbGlzZXIgYXBwbGljYXRpb24uIExvbmctdGVybSBjaGFuZ2VzIGluIG51dHJpZW50IGxvYWRpbmcgYW5kIHN0b2ljaGlvbWV0cnkgaW4gd2F0ZXIgYm9kaWVzIHJlZmxlY3QgY2hhbmdlcyBpbiBhZ3JpY3VsdHVyYWwgcHJhY3RpY2VzIGFuZCB3YXN0ZXdhdGVyIHRyZWF0bWVudC4ifV0%3D.

Eco-tourism, a burgeoning sector valued at INR 20,000 crore nationally, suffers acutely from lake degradation. Iconic sites like Dal Lake and Chilika attract 5-10 million visitors yearly, but persistent scums and odors deter tourists, slashing occupancy by 50-70% during blooms (Nair & Nayak, 2023; Shahnazaryan et al., 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hdnlhIFZpa3JhbWFuIiwiZmFtaWx5IjoiTmFpciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQcmF0ZWVwIEt1bWFyIiwiZmFtaWx5IjoiTmF5YWsifX1dLCJpZHMiOnsiaXNzbiI6WyIyMjk2LTc3NDUiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzODkvZm1hcnMuMjAyMy4xMDg3Mjk2Iiwib3BlbmFsZXgiOiJXNDM4MjcyMzEzMyJ9LCJ0aXRsZSI6IlVuY292ZXJpbmcgd2F0ZXIgcXVhbGl0eSBhbmQgZXZhbHVhdGluZyB2dWxuZXJhYmlsaXRpZXMgb2Ygc21hbGwtc2NhbGUgZmlzaGVyaWVzIGluIENoaWxpa2EgTGFnb29uLCBJbmRpYSIsImFic3RyYWN0IjoiU21hbGwtc2NhbGUgZmlzaGVyaWVzIChTU0ZzKSBwbGF5IGEgc3Ryb25nIHJvbGUgaW4gc3VzdGFpbmluZyBtaWxsaW9ucyBvZiBsaXZlbGlob29kcywgZm9vZCBzZWN1cml0eSwgbnV0cml0aW9uLCBhbmQgaW5jb21lIGdsb2JhbGx5IGJ1dCB0aGUgZmlzaGVycyBlbmdhZ2VkIGluIHRoaXMgc2VjdG9yIHNpbXVsdGFuZW91c2x5IGV4cGVyaWVuY2UgaGlnaCBsZXZlbHMgb2YgdnVsbmVyYWJpbGl0eSBhbmQgcHJvY2Vzc2VzIG9mIG1hcmdpbmFsaXNhdGlvbi4gU2V2ZXJhbCBmYWN0b3JzIGFyZSBhdHRyaWJ1dGVkIHRvIHRoZSBtdWx0aWRpbWVuc2lvbmFsIHZ1bG5lcmFiaWxpdGllcyB0aGUgc21hbGwtc2NhbGUgZmlzaGVycyBleHBlcmllbmNlLCBzcGFubmluZyBib3RoIG5hdHVyYWwgKGUuZy4sIG5hdHVyYWwgZGlzYXN0ZXJzLCBlY29zeXN0ZW0gY2hhbmdlKSBhbmQgYW50aHJvcG9nZW5pYyAoZS5nLiwgcG9saWN5IGNoYW5nZSwgaHlkcm9sb2dpY2FsIGludGVydmVudGlvbnMsIGFxdWFjdWx0dXJlKSBwcmVzc3VyZXMuIFdoaWxlIHRoZXJlIGlzIG11Y2ggbGl0ZXJhdHVyZSBvbiB2YXJpb3VzIG5hdHVyYWwgYW5kIGh1bWFuIGRyaXZlcnMgb2YgdnVsbmVyYWJpbGl0eSBpbiBzbWFsbC1zY2FsZSBmaXNoZXJ5IGNvbW11bml0aWVzLCBhbiBhYnNlbmNlIG9mIHJlc2VhcmNoIGNvbm5lY3RpbmcgdnVsbmVyYWJpbGl0eSB3aXRoIHdhdGVyIHF1YWxpdHkgaXMgZXZpZGVudC4gRmlzaGVyIGNvbW11bml0aWVzIG9mdGVuIHRhbGsgYWJvdXQgZmlzaCBpbiByZWxhdGlvbiB0byB0aGUgaGVhbHRoIG9mIHRoZWlyIGFxdWF0aWMgaGFiaXRhdHMgd2hlcmVpbiB3YXRlciBxdWFsaXR5IGlzIHNlZW4gYXMgYSBrZXkgcGFyYW1ldGVyLiBUaGUgbGluayBiZXR3ZWVuIGhlYWx0aHkgZmlzaCBhbmQgZ29vZCBxdWFsaXR5IHdhdGVyIGhhcyBzaWduaWZpY2FudCBpbXBsaWNhdGlvbnMgZm9yIHN0cm9uZyBhbmQgdmlhYmxlIGZpc2hpbmcgY29tbXVuaXRpZXMuIFRoaXMgcGFwZXIgZXhhbWluZXMgdGhlc2UgbGlua3MgZnVydGhlciBieSBmb2N1c2luZyBvbiB0aGUgbmF0dXJlIG9mIHZ1bG5lcmFiaWxpdGllcyBjYXVzZWQgYnkgd2F0ZXIgcXVhbGl0eSBjaGFuZ2VzIGluIHRoZSBzbWFsbC1zY2FsZSBmaXNoZXJ5IHN5c3RlbSBvZiBDaGlsaWthIExhZ29vbiBpbiBJbmRpYS4gV2UgdW5kZXJ0YWtlIGRldGFpbGVkIGFuYWx5c2lzIG9mIHRoZSBpbnZhc2l2ZSBzaHJpbXAgYXF1YWN1bHR1cmUgYWN0aXZpdGllcyBhbmQgaHlkcm9sb2dpY2FsIGludGVydmVudGlvbnMgZm9yIG9wZW5pbmcgb2YgYSBsYWdvb24gaW5sZXQgd2l0aCB0aGUgQmF5IG9mIEJlbmdhbCBhcyB0d28gZG9taW5hbnQgZHJpdmVycyBhZHZlcnNlbHkgaW1wYWN0aW5nIHdhdGVyIHF1YWxpdHkgYW5kIGluY3JlYXNpbmcgdnVsbmVyYWJpbGl0aWVzIG9mIHRoZSBlbnRpcmUgc21hbGwtc2NhbGUgZmlzaGVyaWVzIHNvY2lhbC1lY29sb2dpY2FsIHN5c3RlbS4gT3VyIGFuYWx5c2lzIHN1Z2dlc3RzIHRoYXQgdGhlcmUgYXJlIHN0cm9uZyBpbnRlcmNvbm5lY3Rpb25zIGJldHdlZW4gY2hhbmdlcyBpbiB3YXRlciBxdWFsaXR5IGFuZCB0aGUgbGV2ZWxzIG9mIHZ1bG5lcmFiaWxpdGllcyBpbiB0aGUgU1NGcyBvZiBDaGlsaWthIExhZ29vbi4gUG9sbHV0YW50cyBzdWNoIGFzIHBlc3RpY2lkZXMsIGFuZCBvcmdhbmljIGNvbXBvdW5kcyBhY2N1bXVsYXRlIGluIGZpc2ggdGlzc3VlcyBhbmQgYWZmZWN0IHRoZWlyIGdyb3d0aCwgcmVwcm9kdWN0aW9uLCBhbmQgb3ZlcmFsbCBoZWFsdGguIFRoaXMgbGVkIHRvIGRlY2xpbmVzIGluIGZpc2ggcG9wdWxhdGlvbnMsIG1ha2luZyBpdCBtb3JlIGRpZmZpY3VsdCBmb3IgZmlzaGVycyB0byBtYWtlIGEgbGl2aW5nLiBJbiBhZGRpdGlvbiB0byBkaXJlY3QgaW1wYWN0cyBvbiBmaXNoIHBvcHVsYXRpb25zLCBwb29yIHdhdGVyIHF1YWxpdHkgYWxzbyBoYXMgaW5kaXJlY3QgZWZmZWN0cyBvbiB0aGUgc29jaWFsIGFuZCBlY29ub21pYyB2dWxuZXJhYmlsaXR5IG9mIFNTRnMuIEZvciBleGFtcGxlLCBjb250YW1pbmF0aW9uIG9mIHdhdGVyIHNvdXJjZXMgbGVkIHRvIHRoZSByZWR1Y2VkIG51bWJlciBvZiBmaXNoIHNwZWNpZXMgcmVkdWNpbmcgdGhlIGFtb3VudCBvZiB0aW1lIGZpc2hlcnMgY2FuIHNwZW5kIG9uIGZpc2hpbmcgYWN0aXZpdGllcy4gVGhpcyBhbHNvIGFmZmVjdGVkIHRoZSBtYXJrZXRhYmlsaXR5IG9mIGZpc2ggcHJvZHVjdHMsIHJlZHVjZWQgaW5jb21lIGFuZCBpbmNyZWFzZWQgcG92ZXJ0eS4gVG8gZnVsbHkgdW5kZXJzdGFuZCB0aGUgaW50ZXJjb25uZWN0aW9ucyBiZXR3ZWVuIHdhdGVyIHF1YWxpdHkgYW5kIHZ1bG5lcmFiaWxpdHkgaW4gU1NGcyBpbiBDaGlsaWthIExhZ29vbiwgaXQgaXMgaW1wb3J0YW50IHRvIGNvbnNpZGVyIGJvdGggZW52aXJvbm1lbnRhbCBhbmQgc29jaWFsIGZhY3RvcnMsIGFzIHdlbGwgYXMgdGhlIGNvbXBsZXggZmVlZGJhY2sgbG9vcHMgYmV0d2VlbiB0aGVzZSBmYWN0b3JzLiBUaGUgc3R1ZHkgaGVscHMgaW4gYnJpZGdpbmcgYSBjcnVjaWFsIGdhcCBpbiBvdXIgdW5kZXJzdGFuZGluZyBvZiB0aGUgcm9sZSBvZiB3YXRlciBxdWFsaXR5IGluIHZ1bG5lcmFiaWxpdHkgYW5hbHlzaXMgd2l0aGluIHJlc291cmNlIGRlcGVuZGVudCBjb21tdW5pdGllcy4gV2UgY29uY2x1ZGUgd2l0aCBrZXkgaW5zaWdodHMgb24gcG9zc2libGUgY29waW5nIHJlc3BvbnNlcyBhbmQgYWRhcHRpdmUgY2FwYWNpdHkgbmVjZXNzYXJ5IGZvciB0aGUgc21hbGwtc2NhbGUgZmlzaGVyaWVzIGNvbW11bml0aWVzIHRvIHRyYW5zaXRpb24gdG93YXJkIHZpYWJpbGl0eS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMzM4OS9mbWFycy4yMDIzLjEwODcyOTYiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vd3d3LmZyb250aWVyc2luLm9yZy9hcnRpY2xlcy8xMC4zMzg5L2ZtYXJzLjIwMjMuMTA4NzI5Ni9wZGYiLCJpc3N1ZWREYXRlIjoiMjAyMy0wNi0yOSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZyb250aWVycyBpbiBNYXJpbmUgU2NpZW5jZSIsInZvbHVtZSI6MTAsImltcGFjdEZhY3RvciI6IjQuMjgifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRnJvbnRpZXJzIE1lZGlhIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo1MSwiY2l0ZWRCeUNvdW50Ijo1fX0sInF1b3RlIjoiV2F0ZXIgcXVhbGl0eSBvZiBDaGlsaWthIExhZ29vbiBoYXMgZGVncmFkZWQgZHVlIHRvIGFudGhyb3BvZ2VuaWMgaW50ZXJ2ZW50aW9ucyBhbmQgbmF0dXJhbCBmb3JjZXMgb2YgY2hhbmdlLiBWYXJpb3VzIGFjdGl2aXRpZXMgaW4gdGhlIENoaWxpa2EgTGFnb29uLCBzdWNoIGFzIGluZHVzdHJpYWwgd2FzdGV3YXRlciBkaXNwb3NhbCwgc2V3YWdlIGR1bXBpbmcsIGFxdWFjdWx0dXJlLCBoeWRyb2xvZ2ljYWwgaW50ZXJ2ZW50aW9ucywgYW5kIGN5Y2xvbmVzLCBhcmUgcHJvZHVjaW5nIHNhbGluaXR5IGNoYW5nZXMsIHNlZGltZW50IGRlcG9zaXRpb24sIG51dHJpZW50IGVucmljaG1lbnQsIGV1dHJvcGhpY2F0aW9uLCBhbmQgZGVhZCB6b25lcywgYXMgc3RhdGVkLiBGaXNoIHN1cnZpdmFsIGlzIGF0IHJpc2sgYXMgYSByZXN1bHQgb2YgdGhlIHNpZ25pZmljYW50IHJlZHVjdGlvbiBpbiBlbnZpcm9ubWVudGFsIGNpcmN1bXN0YW5jZXMsIHdoaWNoIGRpbWluaXNoZXMgZmlzaGVybWVu4oCZcyBpbmNvbWUgYW5kIHB1c2hlcyB0aGVtIG91dCBvZiBzb2NpZXR5LiBUaGVzZSBjYXVzZXMgYWZmZWN0aW5nIHRoZSB3YXRlciBxdWFsaXR5IG9mIHRoZSBMYWdvb24gZGVwaWN0IG1hbnkgZmFjZXMgb2YgdnVsbmVyYWJpbGl0eSBpbiBTU0YgY29tbXVuaXRpZXMuIEFzIHRoZSBDaGlsaWthIExhZ29vbiBlbnZpcm9ubWVudCBpcyBleHBlY3RlZCB0byB1bmRlcmdvIGluY3JlYXNpbmdseSBkcmFzdGljIGNoYW5nZXMgKHN1Y2ggYXMgY2F0Y2ggY29tcG9zaXRpb24sIGNhdGNoIGNhcGFjaXR5LCBiaW9kaXZlcnNpdHkgdmFyaWF0aW9ucywgd2F0ZXIgcXVhbGl0eSwgYW5kIGZpc2hlcnkgcmV2ZW51ZXMpLCBpbnRlcm5hdGlvbmFsIHBvbGljeSBoYXMgYmVjb21lIG1vcmUgaW50ZXJlc3RlZCBpbiBhZGFwdGF0aW9uIG1lYXN1cmVzLiBHaXZlbiBob3cgZGl2ZXJzZSBTU0YgZWNvc3lzdGVtcyBhcmUgZXhwZWN0ZWQgdG8gYmUgYmV0d2VlbiBjb250aW5lbnRzLCB0aGVyZSBpcyBubyDigJxvbmUgc2l6ZSBmaXRzIGFsbOKAnSBhcHByb2FjaC4gVGhlIHBhcGVyIG91dGxpbmVkIHNldmVyYWwgY29waW5nIGFuZCBhZGFwdGF0aW9uIG1lY2hhbmlzbXMgdG8gaW1wcm92ZSB3YXRlciBxdWFsaXR5IGFuZCBjb250aW51ZSBmaXNoaW5nIGluIGEgc3VzdGFpbmFibGUgd2F5LiBBZGFwdGF0aW9uIHN0cmF0ZWdpZXMgY2FuIG9jY2FzaW9uYWxseSBoYXZlIHVuZm9yZXNlZW4gY29uc2VxdWVuY2VzIGZvciBmaXNoZXJpZXMgYW5kIGZpc2hpbmcgY29tbXVuaXRpZXMuIEFueSB1bmV4cGVjdGVkIGNvbnNlcXVlbmNlcyBvZiBhZGFwdGF0aW9uIGNhbiBiZSBtaXRpZ2F0ZWQgdGhyb3VnaCBsb25nLXRlcm0gcGxhbm5pbmcgYW5kIGlkZW50aWZpY2F0aW9uIG9mIGZ1dHVyZSBvcHRpb25zIHRocm91Z2ggc2NlbmFyaW8gYW5hbHlzaXMuIFRoZSBmaW5kaW5ncyBvZiB0aGlzIHBhcGVyIGNhbiBiZSB1dGlsaXplZCB0byBkZXZlbG9wIHBvbGljaWVzIHRvIHByb3RlY3Qgd2F0ZXIgcXVhbGl0eSBhbmQgU1NGcyBpbiBhIHZhcmlldHkgb2YgY29hc3RhbCBsYWdvb24gc2V0dGluZ3MuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjIyLjI4NTAzNjA4NzAzNjEzMywibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ5MzA1MzQzNjI3OTMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjIzLjc3MzUxOTUxNTk5MTIxLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDgzMzY0MTA1MjI0NjEsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjI1LjI1NTU4NDcxNjc5Njg3NSwibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ4OTU1OTE3MzU4Mzk4NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjYuNzQ0MDY4MTQ1NzUxOTUzLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNTA2ODA1NDE5OTIxODc1LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyOC4yMjYxMzMzNDY1NTc2MTcsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40OTA0NjMyNTY4MzU5NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjkuNzE0NjE2Nzc1NTEyNjk1LCJsZWZ0Ijo4LjMzMTU5MjU1OTgxNDQ1Mywid2lkdGgiOjQwLjQ5NDA5ODY2MzMzMDA4LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozMS4xOTY2ODM4ODM2NjY5OTIsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40OTQzOTIzOTUwMTk1MywiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MzIuNjg1MTY1NDA1MjczNDQsImxlZnQiOjguMzMxNTk2Mzc0NTExNzE5LCJ3aWR0aCI6NDAuNDg3NDQ1ODMxMjk4ODMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM0LjE2NzIzMjUxMzQyNzczNCwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40OTc3NDU1MTM5MTYwMTYsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM1LjY1NTcxNTk0MjM4MjgxLCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ5ODYzODE1MzA3NjE3LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozNy4xMzc3ODMwNTA1MzcxMSwibGVmdCI6MTAuNjg1MTg2Mzg2MTA4Mzk4LCJ3aWR0aCI6MzguMTQxMTA1NjUxODU1NDcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM4LjYyNjI2NjQ3OTQ5MjE5LCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4NzE1MjA5OTYwOTM3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDAuMTA4MzMzNTg3NjQ2NDg0LCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4OTg1NjcxOTk3MDcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQxLjU5NjgxMzIwMTkwNDMsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDg4NjY2NTM0NDIzODMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQzLjA3ODg4MDMxMDA1ODU5NCwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40ODE4NjExMTQ1MDE5NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDQuNTY3MzYzNzM5MDEzNjcsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDgzMDU4OTI5NDQzMzYsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQ2LjA0OTQzMDg0NzE2Nzk3LCJsZWZ0Ijo4LjMzMzEwODkwMTk3NzUzOSwid2lkdGgiOjQwLjUwMDc0NzY4MDY2NDA2LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo0Ny41Mzc5MTQyNzYxMjMwNSwibGVmdCI6OC4zMzMxMDg5MDE5Nzc1MzksIndpZHRoIjo0MC40OTU2MDU0Njg3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDkuMDE5OTgxMzg0Mjc3MzQ0LCJsZWZ0Ijo4LjMzMzEwNjA0MDk1NDU5LCJ3aWR0aCI6NDAuNTA2ODE2ODY0MDEzNjcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUwLjUwODQ2NDgxMzIzMjQyLCJsZWZ0Ijo4LjMzMzEwNjA0MDk1NDU5LCJ3aWR0aCI6NDAuNDg4NTA2MzE3MTM4NjcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUxLjk5MDUyODEwNjY4OTQ1LCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDk3NzI2NDQwNDI5NjksImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUzLjQ3OTAxMTUzNTY0NDUzLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6MzMuMTU5NDQ2NzE2MzA4NTk0LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo1NC45NjEwNzg2NDM3OTg4MywibGVmdCI6MTAuNjgzNjcwMDQzOTQ1MzEyLCJ3aWR0aCI6MzguMTM2NTk2Njc5Njg3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NTYuNDQ5NTYyMDcyNzUzOTA2LCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDg3MTQ0NDcwMjE0ODQ0LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEzIn19LHsib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkdheWFuZSIsImZhbWlseSI6IlNoYWhuYXphcnlhbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNYXJ0aW4iLCJmYW1pbHkiOiJTY2h1bHR6ZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLYXJzdGVuIiwiZmFtaWx5IjoiUmlua2UifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQmFyZHVraCIsImZhbWlseSI6IkdhYnJpZWx5YW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIxMTI5LTU3NjciLCIxNzIzLTg2MzMiXSwiaXNibiI6W10sImRvaSI6IjEwLjQwODEvamxpbW5vbC4yMDIyLjIxNjgiLCJvcGVuYWxleCI6Ilc0Mzg4OTc3MDMyIn0sInRpdGxlIjoiTGFrZSBTZXZhbi4gUGFzdCwgcHJlc2VudCwgYW5kIGZ1dHVyZSBzdGF0ZSBvZiBhIHVuaXF1ZSBhbHBpbmUgbGFrZSIsImFic3RyYWN0IjoiTGFrZSBTZXZhbiwgYSBsYXJnZSwgZGVlcCwgYWxwaW5lIGxha2UgaW4gdGhlIExlc3NlciBDYXVjYXN1cyBpcyB0aGUgZm9jdXMgb2YgdGhpcyBTcGVjaWFsIElzc3VlIG9mIHRoZSBKb3VybmFsIG9mIExpbW5vbG9neS4gSXQgd2FzIGFuIG91dHN0YW5kaW5nIGVjb3N5c3RlbSAxMDAgeWVhcnMgYWdvIGNoYXJhY3RlcmlzZWQgYnkgZXhjZWxsZW50IHdhdGVyIHF1YWxpdHksIHJpY2ggYmlvZGl2ZXJzaXR5IHdpdGggYSBoaWdoIGxldmVsIG9mIGVuZGVtaXNtLCB3aWRlLXJhbmdpbmcgYmVkcyBvZiBtYWNyb3BoeXRlcyBhbG9uZyB0aGUgc2hvcmVzIGFuZCBhIHByb2R1Y3RpdmUgYW5kIHN1c3RhaW5hYmxlIGZpc2ggcHJvZHVjdGlvbi4gRHVlIHRvIGl0cyBiZWF1dHksIG5hdHVyYWwgaGlzdG9yeSwgYW5kIGNvbnRyaWJ1dGlvbnMgdG8gc29jaWFsIGFuZCBlY29ub21pYyB3ZWxmYXJlIGl0IGlzIGFsc28gYSBjdWx0dXJhbCBoZXJpdGFnZSBmb3IgdGhlIEFybWVuaWFuIE5hdGlvbiBpbmNsdWRpbmcgaXRzIGxhcmdlIGRpYXNwb3JhLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC40MDgxL2psaW1ub2wuMjAyMi4yMTY4IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5qbGltbm9sLml0L2luZGV4LnBocC9qbGltbm9sL2FydGljbGUvZG93bmxvYWQvMjE2OC8yMDA3IiwiaXNzdWVkRGF0ZSI6IjIwMjMtMTEtMjMiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA5IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJKb3VybmFsIG9mIExpbW5vbG9neSIsInZvbHVtZSI6ODEsImltcGFjdEZhY3RvciI6IjIuMDEifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiUEFHRVByZXNzIChJdGFseSkifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjM1LCJjaXRlZEJ5Q291bnQiOjJ9fSwicXVvdGUiOiJDbGltYXRlIHdhcm1pbmcgY2F1c2VzIGNoYW5nZXMgaW4gdGhlIHRoZXJtYWwgcmVnaW1lIG9mIGxha2VzLCBpLmUuLCB0aGUgdHlwaWNhbCB0ZW1wZXJhdHVyZSBzdHJ1Y3R1cmVzIChXb29sd2F5IGFuZCBNZXItY2hhbnQsIDIwMTkpIGFuZCB0aGUgdGltaW5nIG9mIG1peGluZyBhbmQgc3RyYXRpZmljYXRpb24gKEtyYWVtZXIgZXQgYWwuLCAyMDIxKSwgdGhlIGR1cmF0aW9uIG9mIHN1bW1lciBzdHJhdGlmaWNhdGlvbiAgYW5kIGljZSBjb3ZlciAoUm9nb3JhIGV0IGFsLiwgMjAxODsgU2hhcm1hIGV0IGFsLiwgMjAxOSkgYW5kLCBldmVudHVhbGx5LCBpdCBpcyBhY2NlbGVyYXRpbmcgaW50ZXJuYWwgbnV0cmllbnQgbG9hZGluZyAoTm9ydGggZXQgYWwuLCAyMDE0KSBhbmQgaW1wYWN0aW5nIHRoZSBlbnRpcmUgbGFrZSBlY29zeXN0ZW0gIChTY2h3ZWZlbCBldCBhbC4sIDIwMTk7IFdvb2x3YXkgZXQgYWwuLCAyMDIyKS4gUmVjZW50IHN0dWRpZXMgd29ybGR3aWRlIGRvY3VtZW50ZWQgdW5lcXVpdm9jYWxseSB0aGF0IHRoZSB0eXBpY2FsIHN0cmVzc29ycyBvZiBsYWtlcyBpbnRlcmFjdCwgcG90ZW50aWFsbHkgbGVhZC1pbmcgdG8gZW5mb3JjZWQgbmVnYXRpdmUgY29uc2VxdWVuY2VzLiBUaGVyZSBpcyBjb25zaWRlci1hYmxlIGV2aWRlbmNlIHRoYXQgY2xpbWF0ZSBjaGFuZ2UgaXMgZXhhY2VyYmF0aW5nICBldXRyb3BoaWNhdGlvbiBhbmQgSEFCcyAoUGVhcmwgYW5kIEh1aXNtYW4sIDIwMDg7IFdvb2wtd2F5IGV0IGFsLiwgMjAyMDsgTWVlcmhvZmYgZXQgYWwuLCAyMDIyKSBhbmQgb3RoZXIgZWZmZWN0cyAga25vd24gZnJvbSBldXRyb3BoaWNhdGlvbiBsaWtlIGh5cG94aWEgYW5kIGFub3hpYSBvZiB0aGUgIGh5cG9saW1uaW9uIG9mIHN0cmF0aWZpZWQgbGFrZXMgKEZ1a3VzaGltYSBldCBhbC4sIDIwMTc7ICBSb2dvcmEgZXQgYWwuLCAyMDE4OyBKYW5lIGV0IGFsLiwgMjAyMSkuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NTQuMjUwMjMyNjk2NTMzMiwibGVmdCI6Ny4xNDMzOTMwMzk3MDMzNjksIndpZHRoIjozOC40OTkyNjc1NzgxMjUsImhlaWdodCI6MS4zOTUyNjMzMTQyNDcxMzEzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6MjUuNTIyNzkyODE2MTYyMTEsImxlZnQiOjQ5Ljk5OTE0NTUwNzgxMjUsIndpZHRoIjozOC41MDYyOTQyNTA0ODgyOCwiaGVpZ2h0IjoxMi4xMjk3NzUwNDczMDIyNDZ9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjozOS4zMjI2ODUyNDE2OTkyMiwibGVmdCI6NTIuMzgxMjM3MDMwMDI5Mywid2lkdGgiOjM2LjEyNDQ5NjQ1OTk2MDk0LCJoZWlnaHQiOjEuMzk1MjYzMzE0MjQ3MTMxM319LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjQwLjg1NjAxNDI1MTcwODk4NCwibGVmdCI6NTAuMDAwODIzOTc0NjA5Mzc1LCJ3aWR0aCI6MzguMDg0NDk1NTQ0NDMzNTk0LCJoZWlnaHQiOjEuMzk1MjYzMzE0MjQ3MTMxM319LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjQyLjM4OTMzNTYzMjMyNDIyLCJsZWZ0Ijo1MC4wMDA4MjM5NzQ2MDkzNzUsIndpZHRoIjozOC4wNDg1NTcyODE0OTQxNCwiaGVpZ2h0IjoxLjM5NTI2MzMxNDI0NzEzMTN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo0My45MjI2NjA4Mjc2MzY3MiwibGVmdCI6NTAuMDAwODIzOTc0NjA5Mzc1LCJ3aWR0aCI6MzguNTA1NTEyMjM3NTQ4ODMsImhlaWdodCI6MS4zOTUyNjMzMTQyNDcxMzEzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NDUuNDU1OTg2MDIyOTQ5MjIsImxlZnQiOjUwLjAwMDgyMzk3NDYwOTM3NSwid2lkdGgiOjM4LjA4MDc4MDAyOTI5Njg3NSwiaGVpZ2h0IjoxLjM5NTI2MzMxNDI0NzEzMTN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo0Ni45OTA1NjYyNTM2NjIxMSwibGVmdCI6NTAuMDAwODIzOTc0NjA5Mzc1LCJ3aWR0aCI6MzguNDk5OTQ2NTk0MjM4MjgsImhlaWdodCI6MS4zOTUyNjMzMTQyNDcxMzEzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NDguNTIyNjI4Nzg0MTc5NjksImxlZnQiOjUwLjAwMDgyMzk3NDYwOTM3NSwid2lkdGgiOjM4LjUwMzExMjc5Mjk2ODc1LCJoZWlnaHQiOjEuMzk1MjYzMzE0MjQ3MTMxM319LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjUwLjA1NzIxMjgyOTU4OTg0NCwibGVmdCI6NTAuMDAwODIzOTc0NjA5Mzc1LCJ3aWR0aCI6MzguNDk4OTM5NTE0MTYwMTU2LCJoZWlnaHQiOjEuMzk1MjYzMzE0MjQ3MTMxM319LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjUxLjU5MDUzODAyNDkwMjM0NCwibGVmdCI6NTAuMDAwODI3Nzg5MzA2NjQsIndpZHRoIjozOC41MDEwNzk1NTkzMjYxNywiaGVpZ2h0IjoxLjM5NTI2MzMxNDI0NzEzMTN9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIxIn19XQ%3D%3D. In Bellandur, frothing crises halved houseboat revenues in Bengaluru's tourism circuit, with cleanup costs burdening local operators (Paerl, 2018)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhhbnMgVy4iLCJmYW1pbHkiOiJQYWVybCJ9fV0sImlkcyI6eyJpc3NuIjpbXSwiaXNibiI6W10sImRvaSI6IjEwLjE3NjE1L3luNTUtYjQzOCIsIm9wZW5hbGV4IjoiVzQzMDcwNjM3MzkifSwidGl0bGUiOiJNaXRpZ2F0aW5nIHRveGljIHBsYW5rdG9uaWMgY3lhbm9iYWN0ZXJpYWwgYmxvb21zIGluIGFxdWF0aWMgZWNvc3lzdGVtcyBmYWNpbmcgaW5jcmVhc2luZyBhbnRocm9wb2dlbmljIGFuZCBjbGltYXRpYyBwcmVzc3VyZXMiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTc2MTUveW41NS1iNDM4IiwicmVzb3VyY2VVcmwiOiJodHRwczovL2Nkci5saWIudW5jLmVkdS9kb3dubG9hZHMvd2gyNDcyOTBnIiwiaXNzdWVkRGF0ZSI6IjIwMTgtMDEtMDEiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA2IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJDYXJvbGluYSBEaWdpdGFsIFJlcG9zaXRvcnkgKFVuaXZlcnNpdHkgb2YgTm9ydGggQ2Fyb2xpbmEgYXQgQ2hhcGVsIEhpbGwpIiwiaW1wYWN0RmFjdG9yIjoiMC4xMCJ9LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJVbml2ZXJzaXR5IG9mIE5vcnRoIENhcm9saW5hIGF0IENoYXBlbCBIaWxsIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjowLCJjaXRlZEJ5Q291bnQiOjB9fSwicXVvdGUiOiJUaGlzIGxlYWRzIHRvIGNvbnRpbnVpbmcgZGVtYW5kIGZvciBOIHN1cHBsaWVzIHRvIHN1c3RhaW4gcHJpbWFyeSBwcm9kdWN0aW9uIGFuZCBwcm9tb3RlIGV1dHJvcGhpY2F0aW9uIFszOCwzOV0sICgyKSBhIGxlbmd0aHkgbGVnYWN5IG9mIFAgbG9hZGluZyB0aGF0IGlzIHN0b3JlZCBpbiBzZWRpbWVudHMsIHdoaWNoIGlzIHJlYWRpbHkgcmVjeWNsZWQgYW5kIHN1cHBvcnRzIGV1dHJvcGhpY2F0aW9uLCAoMykgTjIgZml4YXRpb24sIHdoaWxlIHByb3ZpZGluZyDigJxuZXfigJ0gTiwgZmFpbHMgdG8gbWVldCBlY29zeXN0ZW0tc2NhbGUgTiByZXF1aXJlbWVudHMgWzQwLDQxXS4gRnJvbSBhIG51dHJpZW50IG1hbmFnZW1lbnQgcGVyc3BlY3RpdmUsIHRoaXMgbWVhbnMgdGhhdCBjb250aW51ZWQgZXh0ZXJuYWwgbG9hZGluZyBvZiBOIHBsYXlzIGEgY3JpdGljYWwgcm9sZSBpbiBhY2NlbGVyYXRpbmcgZXV0cm9waGljYXRpb24gYW5kIHByb21vdGluZyBhbGdhbCBibG9vbXMgWzM2XS4gT25lIHRyb3VibGluZyBpbmRpY2F0b3Igb2YgTi1kcml2ZW4gZXV0cm9waGljYXRpb24gYW5kIEN5YW5vSEFCcyBpbiBmcmVzaHdhdGVyIHN5c3RlbXMgaXMgdGhlIGdlb2dyYXBoaWMgZXhwYW5zaW9uIGFuZCBpbmNyZWFzZSBmcmVxdWVuY3kgb2YgdGhlIG5vbi1OMiBmaXhpbmcgdG94aW4tcHJvZHVjaW5nIGdlbnVzIE1pY3JvY3lzdGlzIFs0Ml0sIGVzcGVjaWFsbHkgaW4gbGFyZ2UgbGFrZXMgKGUuZy4sIExha2UgRXJpZSwgQ2FuYWRhLVVTQSwgTGFrZSBLYXN1bWFndXJhLCBKYXBhbiwgTGFrZSBPa2VlY2hvYmVlLCBGbG9yaWRhLCBVU0EsIExha2UgVGFpaHUsIENoaW5hKS4gVGhpcyBnZW51cywgYXMgd2VsbCBhcyBhbm90aGVyIHBsYW5rdG9uaWMgbm9uLU4yIGZpeGluZyBDeWFub0hBQiBQbGFua3RvdGhyaXgsIGFuZCBzZXZlcmFsIGJlbnRoaWMgYW5hbG9ndWUgYmxvb20gdGF4YSAoZS5nLiwgbm9uLU4yIGZpeGluZyBMeW5nYnlhIGFuZCBPc2NpbGxhdG9yaWEgc3BwLiksIHNlcnZlIGFzIGluZGljYXRvcnMgb2YgTi1vdmVyLWVucmljaG1lbnQgWzM2LDQyXS4gVGhlIHByb2xpZmVyYXRpb24gb2YgdGhlc2Ugbm9uLU4yIGZpeGluZyBDeWFub0hBQnMgaXMgYW4gYWRkaXRpb25hbCBpbmRpY2F0b3IgdGhlIE4gaW5wdXQgcmVkdWN0aW9ucyBhcmUgbmVlZGVkIHRvIHJldmVyc2UgdGhpcyB0cmVuZC4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo3Ny40NDYyNTA5MTU1MjczNCwibGVmdCI6MTYuNDI4NTE0NDgwNTkwODIsIndpZHRoIjo3MC43MTQ3MjE2Nzk2ODc1LCJoZWlnaHQiOjEuNDUwNzk2NDg0OTQ3MjA0Nn19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjc5LjA1MDk3OTYxNDI1NzgxLCJsZWZ0IjoxMi44NTcwNjEzODYxMDgzOTgsIndpZHRoIjo3NC40OTQ5ODc0ODc3OTI5NywiaGVpZ2h0IjoxLjQ1MDc5NjQ4NDk0NzIwNDZ9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo4MC42NTU1ODYyNDI2NzU3OCwibGVmdCI6MTIuODAxOTYxODk4ODAzNzExLCJ3aWR0aCI6NzQuMzQwMDE5MjI2MDc0MjIsImhlaWdodCI6MS40NTA3OTY0ODQ5NDcyMDQ2fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6ODIuMjYwMTg1MjQxNjk5MjIsImxlZnQiOjEyLjg1NzA2MTM4NjEwODM5OCwid2lkdGgiOjc0LjI4NTE4Njc2NzU3ODEyLCJoZWlnaHQiOjEuNjY2OTc3NjQzOTY2Njc0OH19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjgzLjg2NDkwNjMxMTAzNTE2LCJsZWZ0IjoxMi44NTcwNjEzODYxMDgzOTgsIndpZHRoIjo3NC4yODUwMTg5MjA4OTg0NCwiaGVpZ2h0IjoxLjQ1MDc5NjQ4NDk0NzIwNDZ9fSx7InBhZ2VOdW1iZXIiOjUsImJib3giOnsidG9wIjo4NS40Njk1MTI5Mzk0NTMxMiwibGVmdCI6MTIuODU3MDYxMzg2MTA4Mzk4LCJ3aWR0aCI6NzEuMTA0OTg4MDk4MTQ0NTMsImhlaWdodCI6MS40NTA3OTY0ODQ5NDcyMDQ2fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6ODcuMDc0MjM0MDA4Nzg5MDYsImxlZnQiOjE2LjQyODUxNDQ4MDU5MDgyLCJ3aWR0aCI6NzAuNzE0MTcyMzYzMjgxMjUsImhlaWdodCI6MS40NTA3OTY0ODQ5NDcyMDQ2fX0seyJwYWdlTnVtYmVyIjo1LCJiYm94Ijp7InRvcCI6ODguNjc4ODQwNjM3MjA3MDMsImxlZnQiOjEyLjg1NzA2MTM4NjEwODM5OCwid2lkdGgiOjc0LjI4NjAyNjAwMDk3NjU2LCJoZWlnaHQiOjEuNjY2OTc3NjQzOTY2Njc0OH19LHsicGFnZU51bWJlciI6NSwiYmJveCI6eyJ0b3AiOjkwLjI4MzQzOTYzNjIzMDQ3LCJsZWZ0IjoxMi44NTcwNjEzODYxMDgzOTgsIndpZHRoIjo3NC4yODU0MTU2NDk0MTQwNiwiaGVpZ2h0IjoxLjQ3NjgzMDQ4MjQ4MjkxMDJ9fSx7InBhZ2VOdW1iZXIiOjYsImJib3giOnsidG9wIjoxMC41MDY1MTM1OTU1ODEwNTUsImxlZnQiOjEyLjg1NzA2MTM4NjEwODM5OCwid2lkdGgiOjc0LjIwMjAxODczNzc5Mjk3LCJoZWlnaHQiOjEuNTg2NjA4NDA5ODgxNTkxOH19LHsicGFnZU51bWJlciI6NiwiYmJveCI6eyJ0b3AiOjEyLjExMTIzNzUyNTkzOTk0MSwibGVmdCI6MTIuODU3MDYxMzg2MTA4Mzk4LCJ3aWR0aCI6NzQuMjg1Njc1MDQ4ODI4MTIsImhlaWdodCI6MS42NjY5ODEzMzk0NTQ2NTA5fX0seyJwYWdlTnVtYmVyIjo2LCJiYm94Ijp7InRvcCI6MTMuNzE1ODM3NDc4NjM3Njk1LCJsZWZ0IjoxMi44NTcwNjEzODYxMDgzOTgsIndpZHRoIjo3NC4yODU5NzI1OTUyMTQ4NCwiaGVpZ2h0IjoxLjQ3NjgzNDE3Nzk3MDg4NjJ9fSx7InBhZ2VOdW1iZXIiOjYsImJib3giOnsidG9wIjoxNS4zMjA1NjE0MDg5OTY1ODIsImxlZnQiOjEyLjg1NzA2MTM4NjEwODM5OCwid2lkdGgiOjc0LjI4NTEwMjg0NDIzODI4LCJoZWlnaHQiOjEuNjY2ODY1MzQ4ODE1OTE4fX0seyJwYWdlTnVtYmVyIjo2LCJiYm94Ijp7InRvcCI6MTYuOTI1MTYzMjY5MDQyOTcsImxlZnQiOjEyLjg1NzA2MTM4NjEwODM5OCwid2lkdGgiOjc0LjI4NTM1NDYxNDI1NzgxLCJoZWlnaHQiOjEuNjY2OTgxMzM5NDU0NjUwOX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjYifX1d. Angling and birdwatching economies collapse under hypoxic barrenness, as avian diversity wanes and fish vanish, eroding biodiversity-based attractions (Shahbaz et al., 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1kIiwiZmFtaWx5IjoiU2hhaGJheiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBaGJhciIsImZhbWlseSI6IkFsYW0ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTW9oYW1tYWQgTWFzcm9vciIsImZhbWlseSI6IlphZmFyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1vaGFtbWVkIEFhc2lmIiwiZmFtaWx5IjoiU3VsYWltYW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW51cG1hIiwiZmFtaWx5IjoiS3VtYXJpIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkcuIEsuIiwiZmFtaWx5IjoiU2hhcm1hIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNoYWhsYSIsImZhbWlseSI6Illhc21pbiJ9fV0sImlkcyI6eyJpc3NuIjpbIjEyMTUtMTYxMCIsIjIwNjEtOTU4OCJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMjQ3OC9vcmh1LTIwMjMtMDAxNiIsIm9wZW5hbGV4IjoiVzQzODk1MTAwMzcifSwidGl0bGUiOiJDb21wYXJpc29uIG9mIGF2aWFuIGRpdmVyc2l0eSBiZXR3ZWVuIG1hbmFnZWQgYW5kIHVubWFuYWdlZCB3ZXRsYW5kcyBpbiBQYXRuYSwgQmloYXIsIEluZGlhIiwiYWJzdHJhY3QiOiJBYnN0cmFjdCBXZSBjb21wYXJlZCB0aGUgYmlyZCBkaXZlcnNpdHkgb2YgYSBtYW5hZ2VkIChSYWpkaGFuaSBKYWxhc2hheSkgYW5kIGFuIHVubWFuYWdlZCB3ZXRsYW5kIChNdXN0YWZhcHVyKSBpbiBQYXRuYSwgQmloYXIsIEluZGlhLiBXZSByZWNvcmRlZCBiaXJkIHNwZWNpZXMgYnkgdXNpbmcgcG9pbnQgY291bnRzIGFuZCBmaXhlZC1yb3V0ZSBtb25pdG9yaW5nLiBBdCBSYWpkaGFuaSBKYWxhc2hheSwgYSB0b3RhbCBvZiA3MyBzcGVjaWVzIHdlcmUgcmVjb3JkZWQsIG9mIHdoaWNoIDE1IHNwZWNpZXMgd2VyZSB3aW50ZXIgdmlzaXRvcnMuIFR3byBzcGVjaWVzIHdlcmUgaW4gdGhlIE5lYXItdGhyZWF0ZW5lZCBjYXRlZ29yeTogRmVycnVnaW5vdXMgRHVjayAoQXl0aHlhIG55cm9jYSkgYW5kIEFsZXhhbmRyaW5lIFBhcmFrZWV0IChQc2l0dGFjdWxhIGV1cGF0cmlhKSAsIHdoaWxlIHRoZSBvdGhlcnMgd2VyZSBvZiBsZWFzdCBjb25jZXJuLiBBdCBNdXN0YWZhcHVyIHdldGxhbmQsIDY3IHNwZWNpZXMgd2VyZSByZWNvcmRlZCB3aXRoIDExIHNwZWNpZXMgYXMgd2ludGVyIHZpc2l0b3JzLiBTaW1wc29u4oCZcyBzcGVjaWVzIGRpdmVyc2l0eSBpbmRleCB3YXMgMC43OCBhdCBSYWpkaGFuaSBKYWxhc2hheSwgYW5kIDAuODEgYXQgTXVzdGFmYXB1ciB3ZXRsYW5kLiBUaGUgbnVtYmVyIG9mIHNwZWNpZXMgb2Ygd2FkZXJzIHdhcyAxNyBhdCBNdXN0YWZhcHVyIHdldGxhbmQgYW5kIDExIGF0IFJhamRoYW5pIEphbGFzaGF5LiBUaGUgdG90YWwgbnVtYmVyIG9mIExlc3NlciBXaGlzdGxpbmctZHVjayAoRGVuZHJvY3lnbmEgamF2YW5pY2EpICwgYSByZXNpZGVudCBiaXJkIHdhcyBhbG1vc3QgZG91YmxlICg1MjYpIGF0IE11c3RhZmFwdXIgd2V0bGFuZCB0aGFuIHRoYXQgYXQgUmFqZGhhbmkgSmFsYXNoYXkgKDIzNCkuIFRoZSBuaXRyYXRlIGxldmVsIHdhcyB0d2ljZSBoaWdoZXIgYW5kIHRoZSBwaG9zcGhhdGUgbGV2ZWwgd2FzIGZvdXIgdGltZXMgaGlnaGVyIGF0IE11c3RhZmFwdXIgd2V0bGFuZCB0aGFuIHRob3NlIGF0IFJhamRoYW5pIEphbGFzaGF5LiBUaGUgZGlmZmVyZW5jZSBpbiBzcGVjaWVzIG51bWJlciBhbmQgY29tcG9zaXRpb24gYmV0d2VlbiB0aGUgdHdvIHdldGxhbmRzIG1heSBiZSBhdHRyaWJ1dGVkIHRvIHRoZSBldXRyb3BoaWNhdGlvbiByZXN1bHRpbmcgZnJvbSBoaWdoIG5pdHJhdGUgYW5kIHBob3NwaGF0ZSBsZXZlbHMgYXQgTXVzdGFmYXB1ciB3ZXRsYW5kLiBVbmZvcnR1bmF0ZWx5LCB0aGUgTXVzdGFmYXB1ciB3ZXRsYW5kIGlzIHVuZGVyIHRocmVhdCBkdWUgdG8gaHVtYW4gaW5mbHVlbmNlcy4gVGhlIHByb3RlY3Rpb24gYW5kIHN1c3RhaW5hYmxlIG1hbmFnZW1lbnQgb2YgbmF0dXJhbCB3ZXRsYW5kcyBpcyByZXF1aXJlZCBmb3Igc2F2aW5nIHRoZSBiaW9kaXZlcnNpdHkgb2YgdGhlIGFyZWEuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjI0Nzgvb3JodS0yMDIzLTAwMTYiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vc2NpZW5kby5jb20vcGRmLzEwLjI0Nzgvb3JodS0yMDIzLTAwMTYiLCJpc3N1ZWREYXRlIjoiMjAyMy0xMi0wMSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6Ik9ybmlzIEh1bmdhcmljYSIsImlzc3VlIjoyLCJ2b2x1bWUiOjMxLCJpbXBhY3RGYWN0b3IiOiIwLjkyIn0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTMsImxhc3RQYWdlIjoyOH19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJIdW5nYXJpYW4gT3JuaXRob2xvZ2ljYWwgYW5kIE5hdHVyZSBDb25zZXJ2YXRpb24gU29jaWV0eSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MjcsImNpdGVkQnlDb3VudCI6MX19LCJxdW90ZSI6Ikhvd2V2ZXIsIGhpZ2ggbGV2ZWxzIG9mIGV1dHJvcGhpY2F0aW9uIG1heSBhbHNvIHRyaWdnZXIgdGhlIHByb2xpZmVyYXRpb24gb2YgZ3JlZW4gYWxnYWUgYW5kICBkZWNsaW5lIG9mIG1hY3JvcGh5dGVzIGxlYWRpbmcgdG8gYSBoaWdoLXR1cmJpZGl0eSBzdGF0ZSBvZiB3YXRlciwgaW4gd2hpY2ggZmV3IHNwZWNpZXMgY2FuICBzdXJ2aXZlIChTY2hlZmZlciBldCBhbC4gMTk5MykuIFRvIGFkZHJlc3MgdGhlIGV1dHJvcGhpY2F0aW9uIGFuZCBleGNlc3MgbnV0cmllbnQgaXNzdWVzIGluICBhcXVhdGljIGVjb3N5c3RlbXMsIHBoeXRvcmVtZWRpYXRpb24gb2ZmZXJzIGEgcHJvbWlzaW5nIHNvbHV0aW9uIChMaXUgZXQgYWwuIDIwMjEpLiBBcXVhdGljICBwbGFudHMgbGlrZSBkdWNrd2VlZHMsIHdhdGVyIGxldHR1Y2UgKFBpc3RpYSBzdHJhdGlvdGVzKSwgYW5kIHdhdGVyIGxpbGllcyBoYXZlIGRlbW9uc3RyYXRlZCAgdGhlaXIgYWJpbGl0eSB0byBjb21iYXQgZXV0cm9waGljYXRpb24gYnkgYWJzb3JiaW5nIGFuZCBhc3NpbWlsYXRpbmcgZXhjZXNzIHBob3NwaG9ydXMgYW5kICBuaXRyb2dlbiAoTmFoYXIgJiBIb3F1ZSAyMDIxKS4gSG93ZXZlciwgdGhlc2UgcGxhbnRzIGNhbiBzb21ldGltZXMgZ3JvdyB1bmNvbnRyb2xsYWJseSwgIHBvdGVudGlhbGx5IGNvdmVyaW5nIHRoZSB3YXRlciBzdXJmYWNlcy4gVG8gYXZvaWQgc3VjaCBzY2VuYXJpb3MsIHJlZ3VsYXIgbW9uaXRvcmluZyBpcyBlc3NlbnRpYWwgKExpdSBldCBhbC4gMjAyMSkuIFByb21vdGluZyBwdWJsaWMgYXdhcmVuZXNzIGlzIHRoZSBjb3JuZXJzdG9uZSBmb3IgbW9iaWxpemluZyBzdXBwb3J0IGFuZCBlbnN1cmluZyBlZmZlY3RpdmUgZXV0cm9waGljYXRpb24gbWFuYWdlbWVudC4gQ29udHJvbGxlZCBlcmFkaWNhdGlvbiBvZiBleGNlc3MgIGZsb3JhIHdoZW4gbmVjZXNzYXJ5IGlzIHZpdGFsIHRvIHByZXZlbnQgb3Zlcmdyb3d0aC4gVGhpcyBpbnRlZ3JhdGVkIGFwcHJvYWNoIGNhbiBlZmZlY3RpdmVseSAgcmVkdWNlIGV1dHJvcGhpY2F0aW9uIGluIHRoZSB3YXRlciBib2RpZXMgKFdhbmcgZXQgYWwuIDIwMjIpLiBFYWNoIHNwZWNpZXMgb2Ygd2V0bGFuZC1kZXBlbmRlbnQgYmlyZCBoYXMgYSB1bmlxdWUgYW5kIGNvbXBsZXggc2V0IG9mIG5lZWRzIGZvciB3ZXRsYW5kIGhhYml0YXRzIHRoYXQgbWFrZXMgaXQgZGlmZmljdWx0IHRvIGdlbmVyYWxpemUgYWJvdXQgaG93IGxvc3Mgb3IgZGVncmFkYXRpb24gb2Ygd2V0bGFuZHMgYWZmZWN0cyBiaXJkIHBvcHVsYXRpb25zLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo3OS4yOTU2MzkwMzgwODU5NCwibGVmdCI6MTAuMzAzMDA2MTcyMTgwMTc2LCJ3aWR0aCI6NzkuOTUyNzI4MjcxNDg0MzgsImhlaWdodCI6MS43MDkxOTc4Nzg4Mzc1ODU0fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjgxLjIxMjUzOTY3Mjg1MTU2LCJsZWZ0IjoxMC4zMDMwMDYxNzIxODAxNzYsIndpZHRoIjo3OS45NTkyODE5MjEzODY3MiwiaGVpZ2h0IjoxLjcwOTE5Nzg3ODgzNzU4NTR9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6ODMuMTI5NDQ3OTM3MDExNzIsImxlZnQiOjEwLjMwMzAwNjE3MjE4MDE3Niwid2lkdGgiOjc5Ljk0MjcyNjEzNTI1MzksImhlaWdodCI6MS43MDkxOTc4Nzg4Mzc1ODU0fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjg1LjA0NjM0ODU3MTc3NzM0LCJsZWZ0IjoxMC4zMDMwMDgwNzk1Mjg4MDksIndpZHRoIjo3OS45Mzk0NTMxMjUsImhlaWdodCI6MS43MDkxOTc4Nzg4Mzc1ODU0fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjg2Ljk2MzI1NjgzNTkzNzUsImxlZnQiOjEwLjMwMzAxMDk0MDU1MTc1OCwid2lkdGgiOjc5Ljk0MTg0ODc1NDg4MjgxLCJoZWlnaHQiOjEuNzA5MTk3ODc4ODM3NTg1NH19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo4OC44ODAxNjUxMDAwOTc2NiwibGVmdCI6MTAuMzAzMDEzODAxNTc0NzA3LCJ3aWR0aCI6NzkuOTM3NSwiaGVpZ2h0IjoxLjcwOTE5Nzg3ODgzNzU4NTR9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6OTAuNzk3MDY1NzM0ODYzMjgsImxlZnQiOjEwLjMwMzAxMzgwMTU3NDcwNywid2lkdGgiOjc5LjkzOTY1MTQ4OTI1NzgxLCJoZWlnaHQiOjEuNzA5MTk3ODc4ODM3NTg1NH19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo5Mi43MTM5NzM5OTkwMjM0NCwibGVmdCI6MTAuMzAzMDEzODAxNTc0NzA3LCJ3aWR0aCI6NzkuOTUwMzI1MDEyMjA3MDMsImhlaWdodCI6MS43MDkxOTc4Nzg4Mzc1ODU0fX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjEyLjIwNzk5MDY0NjM2MjMwNSwibGVmdCI6MTAuMzAzMDI0MjkxOTkyMTg4LCJ3aWR0aCI6NzkuOTUyNTE0NjQ4NDM3NSwiaGVpZ2h0Ijo3LjQ1OTkyMjc5MDUyNzM0NH19LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjoxOS44NzU2MjU2MTAzNTE1NjIsImxlZnQiOjEyLjEyMTMyNDUzOTE4NDU3LCJ3aWR0aCI6NzguMTE5MjI0NTQ4MzM5ODQsImhlaWdodCI6MS43MDkxOTc4Nzg4Mzc1ODU0fX0seyJwYWdlTnVtYmVyIjoxMywiYmJveCI6eyJ0b3AiOjIxLjc5MjUzMzg3NDUxMTcyLCJsZWZ0IjoxMC4zMDMwMjkwNjAzNjM3Nywid2lkdGgiOjc5Ljk0OTI0OTI2NzU3ODEyLCJoZWlnaHQiOjEuNzA5MTk3ODc4ODM3NTg1NH19LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjoyMy43MDk0MzY0MTY2MjU5NzcsImxlZnQiOjEwLjMwMzAyOTA2MDM2Mzc3LCJ3aWR0aCI6NzkuOTUyMjcwNTA3ODEyNSwiaGVpZ2h0IjoxLjcwOTE5Nzg3ODgzNzU4NTR9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIyNSJ9fV0%3D. Homogenized communities from invasives diminish scenic appeal, while health scares from dermal exposures curtail recreational use (Paerl et al., 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhhbnMgVy4iLCJmYW1pbHkiOiJQYWVybCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLLkUuIiwiZmFtaWx5IjoiSGF2ZW5zIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik4uUy4iLCJmYW1pbHkiOiJIYWxsIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlQuRy4iLCJmYW1pbHkiOiJPdHRlbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNLiIsImZhbWlseSI6IlpodSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJIYWlodWkiLCJmYW1pbHkiOiJYdSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJHLiIsImZhbWlseSI6IlpodSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJCLiIsImZhbWlseSI6IlFpbiJ9fV0sImlkcyI6eyJpc3NuIjpbXSwiaXNibiI6W10sImRvaSI6IjEwLjE3NjE1LzRua2Qtems5MyIsIm9wZW5hbGV4IjoiVzQzMDcwNjI0MjkifSwidGl0bGUiOiJNaXRpZ2F0aW5nIGEgZ2xvYmFsIGV4cGFuc2lvbiBvZiB0b3hpYyBjeWFub2JhY3RlcmlhbCBibG9vbXM6IENvbmZvdW5kaW5nIGVmZmVjdHMgYW5kIGNoYWxsZW5nZXMgcG9zZWQgYnkgY2xpbWF0ZSBjaGFuZ2UiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTc2MTUvNG5rZC16azkzIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2Nkci5saWIudW5jLmVkdS9kb3dubG9hZHMvMDU3NDIyODM0IiwiaXNzdWVkRGF0ZSI6IjIwMjAtMDEtMDEiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTAxIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJDYXJvbGluYSBEaWdpdGFsIFJlcG9zaXRvcnkgKFVuaXZlcnNpdHkgb2YgTm9ydGggQ2Fyb2xpbmEgYXQgQ2hhcGVsIEhpbGwpIn0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlVuaXZlcnNpdHkgb2YgTm9ydGggQ2Fyb2xpbmEgYXQgQ2hhcGVsIEhpbGwifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjAsImNpdGVkQnlDb3VudCI6MH19LCJxdW90ZSI6IlRoYXQgcmVsYXRpb25zaGlwIGhhcyBzZXJ2ZWQgYXMgdGhlIGZvdW5kYXRpb24gZm9yIHNldHRpbmcgIG51dHJpZW50IGxvYWRpbmcgdGFyZ2V0cyBpbiBsYWtlcyBhcm91bmQgdGhlIHdvcmxkLCBhbmQgY2FyZWZ1bGx5ICBkZXNpZ25lZCBleHBlcmltZW50cyBtYXkgZWx1Y2lkYXRlIHRvIHdoYXQgZXh0ZW50IHRlbXBlcmF0dXJlICBpbmNyZWFzZSBhZmZlY3RzIHRoZSB0aHJlc2hvbGQgYXQgd2hpY2ggdGhlIGZyZXF1ZW5jeSBvZiBvY2N1ciAgcmVuY2Ugb2YgQ3lhbm9IQUJzIHF1aWNrbHkgcmlzZXMgd2l0aCBpbmNyZWFzaW5nIE4gYW5kIFAuIENvbmNsdXNpb25zIFJlY2VudCBzdHVkaWVzIGluZGljYXRlIHRoYXQgdGhlIHN5bmVyZ2lzdGljIGNvbWJpbmF0aW9uIG9mIGFudGhyb3BvZ2VuaWMgbnV0cmllbnQgbG9hZGluZywgcmlzaW5nIHRlbXBlcmF0dXJlcywgZW5oYW5jZWQgIHZlcnRpY2FsIHN0cmF0aWZpY2F0aW9uLCBpbmNyZWFzZWQgcmVzaWRlbmNlIHRpbWUgYW5kIG1vcmUgIGV4dHJlbWUgY2xpbWF0aWMgY29uZGl0aW9ucyBvdmVyYWxsIHdpbGwgZmF2b3VyIGN5YW5vYmFjdGVyaWFsICBkb21pbmFuY2UgaW4gYSB3aWRlIHJhbmdlIG9mIGFxdWF0aWMgZWNvc3lzdGVtcy4gVGhlIGV4cGFuICBzaW9uIG9mIEN5YW5vSEFCcyBoYXMgc2VyaW91cyBjb25zZXF1ZW5jZXMgZm9yIGh1bWFuIHdhdGVyICBzdXBwbGllcywgZmlzaGVyaWVzIGFuZCByZWNyZWF0aW9uYWwgcmVzb3VyY2VzLiBFY29zeXN0ZW0gc2NhbGUgIHBoeXNpY2FsIGNoZW1pY2FsIGJpb3RpYyBtaXRpZ2F0aW9uIHN0cmF0ZWdpZXMgd2lsbCBoYXZlIHRvICBpbmNvcnBvcmF0ZSBudXRyaWVudCAobW9zdCBvZnRlbiBib3RoIE4gYW5kIFApIGlucHV0IHJlZHVjdGlvbnMuICBUaGUgbWFnbml0dWRlcyBvZiB0aGVzZSByZWR1Y3Rpb25zIHdpbGwgbmVlZCB0byBiZSBzeXN0ZW0gIHNwZWNpZmljIGFuZCB3aWxsIGxpa2VseSBuZWVkIHRvIGJlIGFkanVzdGVkIHdpdGggY2hhbmdpbmcgY2xpICBtYXRpYyBjb25kaXRpb25zLiBTcGVjaWZpY2FsbHksIHdhdGVyIHF1YWxpdHkgbWFuYWdlcnMgd2lsbCBoYXZlICB0byBhY2NvbW1vZGF0ZSB3YXJtaW5nIGFuZCBncmVhdGVyIGh5ZHJvbG9naWNhbCB2YXJpYWJpbGl0eSAgYW5kIGV4dHJlbWVuZXNzIHJlc3VsdGluZyBmcm9tIGNsaW1hdGUgY2hhbmdlIGluIGZvcm11bGF0aW5nICB0aGVpciBzdHJhdGVnaWVzLiBBIGtleSBsb25nIHRlcm0gY29udHJvbCB3ZSBjYW4gZXhlcnQgdG8gcmVkdWNlICB0aGUgcmF0ZSBhbmQgZXh0ZW50IG9mIGdsb2JhbCB3YXJtaW5nIGlzIGN1cmJpbmcgZ3JlZW5ob3VzZSBnYXMgIGVtaXNzaW9ucy4gV2l0aG91dCB0aGlzIGVzc2VudGlhbCBzdGVwLCBmdXR1cmUgd2FybWluZyB0cmVuZHMsICBoeWRyb2xvZ2ljYWwgZXh0cmVtZXMgKHJlY29yZCByYWluZmFsbHMgYW5kIGRyb3VnaHRzKSBhbmQgdGhlaXIgIGVmZmVjdHMgb24gYXF1YXRpYyBlY29zeXN0ZW1zIHdpbGwgY29udGludWUgdG8gcHJvbW90ZSBvcHBvciAgdHVuaXN0aWMgQ3lhbm9IQUJzLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjoxNC4xNDkyMzM4MTgwNTQyLCJsZWZ0IjoxMy4xMjg1OTUzNTIxNzI4NTIsIndpZHRoIjozNy4wODIxNzIzOTM3OTg4MywiaGVpZ2h0IjoxLjUxNjYwMjI3Nzc1NTczNzN9fSx7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6MTUuNDUxNTA4NTIyMDMzNjkxLCJsZWZ0IjoxMy4xMjg1NDQ4MDc0MzQwODIsIndpZHRoIjozNy4wODQ4MzEyMzc3OTI5NywiaGVpZ2h0IjoxLjUxNjYwMzIzMTQzMDA1Mzd9fSx7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6MTYuNzUzNzg0MTc5Njg3NSwibGVmdCI6MTMuMTI4NTQyOTAwMDg1NDUsIndpZHRoIjozNy4wODMyMDk5OTE0NTUwOCwiaGVpZ2h0IjoxLjUxNjYwNDE4NTEwNDM3MDF9fSx7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6MTguMDU2MDUzMTYxNjIxMDk0LCJsZWZ0IjoxMy4xMjg1NDQ4MDc0MzQwODIsIndpZHRoIjozNi42MTUwNzQxNTc3MTQ4NDQsImhlaWdodCI6MS41MTY2MDQxODUxMDQzNzAxfX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjE5LjQ2ODM3ODA2NzAxNjYsImxlZnQiOjEzLjEyODU0NDgwNzQzNDA4Miwid2lkdGgiOjM0LjI5MDUzMTE1ODQ0NzI2NiwiaGVpZ2h0IjoxLjI0NTAxMTIxMDQ0MTU4OTR9fSx7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6MjIuMjI2MTQyODgzMzAwNzgsImxlZnQiOjEzLjEyODU1MDUyOTQ3OTk4LCJ3aWR0aCI6Ny42NDk4NzM3MzM1MjA1MDgsImhlaWdodCI6MS4xMzU0ODMxNDU3MTM4MDYyfX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjIzLjczNDg0OTkyOTgwOTU3LCJsZWZ0IjoxMy4xMjg1MzUyNzA2OTA5MTgsIndpZHRoIjozNy4wODkyODY4MDQxOTkyMiwiaGVpZ2h0IjoyNi4wODg2NDAyMTMwMTI2OTV9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIxMSJ9fV0%3D. Operators face compounded hits: forgone bookings, infrastructure decay from algal biofilms, and reputational damage persisting post-blooms (Haxthausen, 2021)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkthcmwgUmFzbXVzIEF1Z3VzdCIsImZhbWlseSI6IkhheHRoYXVzZW4ifX1dLCJpZHMiOnsiaXNzbiI6W10sImlzYm4iOltdLCJtYWciOiIzMTc3MTEzMjA3Iiwib3BlbmFsZXgiOiJXMzE3NzExMzIwNyJ9LCJ0aXRsZSI6IlNlZGltZW50IG1pY3JvYmlhbCBmdWVsIGNlbGxzIGZvciBsYWtlIHJlc3RvcmF0aW9uIiwiYWJzdHJhY3QiOiJFdXRyb3BoaWNhdGlvbiBvZiBsYWtlcyBpcyB1YmlxdWl0b3VzLCBhcyAzMC00MCAlIG9mIHRoZSB3b3JsZOKAmXMgbGFrZXMgYXJlIGV1dHJvcGhpYywgYW5kIGNvbmRpdGlvbnMgY29udGludW91c2x5IGRldGVyaW9yYXRpbmcuIEV1dHJvcGhpY2F0aW9uIGlzIGEgY29uZGl0aW9uIG9mIG92ZXJmZXJ0aWxpemF0aW9uLCB3aGVyZSBhbGwgcmVxdWlyZW1lbnRzIGZvciByYXBpZCBncm93dGggb2YgYWxnYWUgYXJlIG1ldCwgcmVzdWx0aW5nIGluIGFsZ2FsIGJsb29tcy4gVGhpcyBjYXVzZXMgYSBzZWxmLXN1c3RhaW5lZCwgdmljaW91cyBjeWNsZSB3aGVyZSBwb29yIGVjb2xvZ2ljYWwgY29uZGl0aW9ucyBhcmUgbWFpbnRhaW5lZCB1bnRpbCBudXRyaWVudHMgYXJlIGxvc3QgZnJvbSB0aGUgbGFrZSBvdmVyIG1hbnkgeWVhcnMuIExha2UgcmVzdG9yYXRpb24gc2Vla3MgdG8gYnJlYWsgdGhpcyB2aWNpb3VzIGN5Y2xlLiBJbmFkZXF1YXRlIGF2YWlsYWJpbGl0eSBvZiBhbnkgc2luZ2xlIG51dHJpZW50IGluIHdpbGwgbGltaXQgYWxnYWwgZ3Jvd3RoLCB0eXBpY2FsbHkgcGhvc3Bob3J1cyBpcyB0YXJnZXRlZC4gVGhlIHNjb3BlIG9mIHRoaXMgUGhEIHRoZXNpcyB3YXMgdG8gZGV2ZWxvcCwgdGVzdCBhbmQgb3B0aW1pemUgc2VkaW1lbnQgbWljcm9iaWFsIGZ1ZWwgY2VsbHMgKFNNRkMpIGFzIGEgbm92ZWwgbWV0aG9kIGZvciBsYWtlIHJlc3RvcmF0aW9uLiBUaGUgdGhlc2lzIHNwYW5zIHRoZSB0d28gbWFpbiB0b3BpY3Mgb2YgU01GQ3MgYW5kIGxha2UgcmVzdG9yYXRpb24gdGVjaG5pcXVlcy4gTGFrZSByZXN0b3JhdGlvbiB0ZWNobmlxdWVzIGFyZSByZXZpZXdlZCwgcHJvdmlkaW5nIHVuZGVyc3RhbmRpbmcgb2YgdGhlIHVuZGVybHlpbmcgbWVjaGFuaXNtcy4gU01GQ3MgYXJlIGFwcHJvYWNoZWQgZnJvbSBhbiBlbmdpbmVlcmluZyBwZXJzcGVjdGl2ZSBhbmQgYSBmb2N1cyBvbiBsYWtlIHJlbWVkaWF0aW9uLiBUaGUgc3BlY2lmaWMgb2JqZWN0aXZlcyB3ZXJlIHRvIHRlc3QgYSBsb3ctY29zdCBlbGVjdHJvZGUgbWF0ZXJpYWwgZm9yIGl0cyBkdXJhYmlsaXR5LCBwcmFjdGljYWxpdHksIGFuZCBwZXJmb3JtYW5jZS4gUGVyZm9ybWFuY2Ugd2FzIGFzc2Vzc2VkIGFzIHRoZSBhYmlsaXR5IHRvIGxpbWl0IHBob3NwaG9ydXMgcmVsZWFzZSBmcm9tIHNlZGltZW50cywgaW5oaWJpdCBtZXRoYW5lIHJlbGVhc2UsIGFuZCB0aGUgYWRkZWQgYmVuZWZpdHMgb2YgZGVncmFkaW5nIHBvbGx1dGFudHMsIGFuZCBnZW5lcmF0aW5nIGVsZWN0cmljaXR5LiBQcm9vZiBvZiBjb25jZXB0IHdhcyBzaG93biBpbiBsYWJvcmF0b3J5IHJlYWN0b3JzICgxIEwpLCB0aGUgZWZmZWN0IG9mIHVwc2NhbGluZyB0byAyMDAgTCBpbiBuYXR1cmFsIGVudmlyb25tZW50IHdhcyBzdHVkaWVkIGluIG91dGRvb3IgbWVzb2Nvc21zLCBhbmQgdGhlbiB0ZXN0ZWQgYXQgcGlsb3Qgc2NhbGUgaW4gYSBldXRyb3BoaWMgZnJlc2h3YXRlciBsYWtlLiBQaWxvdCBpbnN0YWxsYXRpb25zIHdlcmUgdGhlIGxhcmdlc3QgaW4gdGhlIHdvcmxkIG9mIHRoZWlyIGtpbmQgYW5kIHdlcmUgb3BlcmF0ZWQgZm9yIDIgeWVhcnMuIFN0YWlubGVzcyBzdGVlbCB3YXMgc2VsZWN0ZWQgZm9yIHRlc3RpbmcgYXMgZWxlY3Ryb2RlIG1hdGVyaWFsIGR1ZSB0byBpdHMgbG93IGNvc3QsIGR1cmFiaWxpdHksIGFuZCBhdmFpbGFiaWxpdHkuIFBhaXJzIG9mIGVsZWN0cm9kZXMgd2VyZSBlbWJlZGRlZCBpbiB0aGUgc2VkaW1lbnQgKHRoaXMgaXMgY2FsbGVkIHRoZSBhbm9kZSkgYW5kIHN1c3BlbmRlZCBpbiB0aGUgb3Zlcmx5aW5nIHdhdGVyICh0aGlzIGlzIGNhbGxlZCB0aGUgY2F0aG9kZSkuIFRoZSB0d28gZWxlY3Ryb2RlcyB3ZXJlIGNvbm5lY3RlZCBhY3Jvc3MgYW4gZXh0ZXJuYWwgcmVzaXN0b3Igd2l0aCBlbGVjdHJpY2FsIHdpcmVzLiBNaWNyb2JlcyBpbiB0aGUgc2VkaW1lbnQgdXNlIHRoZSBhbm9kZSBwYWlyIGFzIGVsZWN0cm9uIGFjY2VwdG9yLCBlbmFibGluZyByZXNwaXJhdGlvbiBpbiB0aGUgYW5veGljIHNlZGltZW50LiBFbGVjdHJvbnMgdHJhdmVsIGZyb20gdGhlIHNlZGltZW50IHZpYSB0aGUgZWxlY3RyaWNhbCBjb25uZWN0b3IgdG8gdGhlIG92ZXJseWluZyBjYXRob2RlIHdoZXJlIG94eWdlbiBpbiB0aGUgd2F0ZXIgaXMgdGhlIHRlcm1pbmFsIGVsZWN0cm9uIGFjY2VwdG9yIGNvbXBsZXRpbmcgdGhlIGNpcmN1aXQuIEFzIGFuIGFkZGVkIGJlbmVmaXQsIGRlZ3JhZGF0aW9uIG9mIHBvbGx1dGFudHMgdXNpbmcgU01GQyB3YXMgdGVzdGVkIGluIG1hcmluZSBoYXJib3VyIHNlZGltZW50cywgYnV0IG5vIGNvbmNsdXNpdmUgcmVzdWx0cyBvYnRhaW5lZC4gTGFib3JhdG9yeSByZWFjdG9ycyB3ZXJlIG9wZXJhdGVkIGZvciBtb3JlIHRoYW4gMjAwIGRheXMgdXNpbmcgcGV0cm9sZXVtIGFzIHNvbGUgY2FyYm9uIHNvdXJjZSwgY29uZmlybWluZyBvaWwgZGVncmFkYXRpb24gYnkgU01GQ3MuIEEgcmVtb3ZhbCByYXRlIG9mIDAuOSBnIHBldHJvbGV1bSBtLTMgZC0xIHJlYWN0b3Igdm9sdW1lIHdhcyBlc3RpbWF0ZWQgZnJvbSBtZWFzdXJlZCBlbGVjdHJpY2FsIG91dHB1dC4gVGhpcyB3YXMgZG9uZSBieSBhc3N1bXB0aW9uIG9mIHRoZSBjaGVtaWNhbCByZWFjdGlvbiBvY2N1cnJpbmcsIGZyb20gdGhpcyB0aGUgbnVtYmVyIG9mIGVsZWN0cm9ucyB0cmFuc2ZlcnJlZCBmcm9tIG9pbCB0byB3YXRlciB3YXMga25vd24gYW5kIGNvdWxkIGJlIGNvcnJlbGF0ZWQgdG8gdGhlIG1lYXN1cmVkIGZsb3cgb2YgZWxlY3Ryb25zIChjdXJyZW50KSB0aHJvdWdoIHRoZSBleHRlcm5hbCBjaXJjdWl0LiBTdGFpbmxlc3Mgc3RlZWwgZWxlY3Ryb2RlcyB3ZXJlIG9wZXJhdGVkIGluIGZyZXNod2F0ZXIgbGFrZSBlbnZpcm9ubWVudHMgZm9yIG1vcmUgdGhhbiA0IHllYXJzIHdpdGggbWluaW1hbCBkYW1hZ2UgdG8gdGhlIG1hdGVyaWFsLiBUaGUgZWxlY3Ryb2RlIGlzIHRoZSBtYWluIG1hdGVyaWFsIGNvc3QgaW4gc3VjaCBzeXN0ZW1zLiBIb3dldmVyLCBBSVNJMzE2IHN0YWlubGVzcyBzdGVlbCBzaW1pbGFyIHRvIHdoYXQgd2FzIHVzZWQgaW4gdGhpcyB0aGVzaXMgd2FzIGF0IHRoZSB0aW1lIG9mIHdyaXRpbmcgYXZhaWxhYmxlIGF0IGJ1bGsgY29zdCBvZiAwLjA5MSDigqwgbS0yLCBjb3JyZXNwb25kaW5nIHRvIDkxMCDigqwgYWNyZS0xLCBzaWduaWZpY2FudGx5IGxvd2VyIHRoYW4gb3RoZXIgY29tbW9uIG1hdGVyaWFscy4gQ2F0aG9kZXMgYXQgbGFyZ2Ugc2NhbGUgY291bGQgYmUgbW92ZWQgaG9yaXpvbnRhbGx5IGFuZCB2ZXJ0aWNhbGx5IHdpdGggbWlub3IgaW1wYWN0IHRvIHBlcmZvcm1hbmNlLiBQaWxvdCBzY2FsZSBTTUZDcyBnZW5lcmF0ZWQgMC4zODMgbVcgbS0yIHN1c3RhaW5lZCBhbmQgMjcuNCBtVyBtLTIgcGVhayBvdXRwdXQuIFRoZSBzeXN0ZW0gd2FzIGxpbWl0ZWQgYnkgdGhlIGNhdGhvZGUgc3VyZmFjZSBhcmVhLCBtZWFuaW5nIG11bHRpLWxheWVyIGNhdGhvZGVzIHNob3VsZCBiZSB0ZXN0ZWQgaW4gdGhlIGZ1dHVyZSBpZiBlbGVjdHJpY2FsIHBvd2VyIHByb2R1Y3Rpb24gaXMgdGhlIGdvYWwuIFRoZSBjYXRob2RlIHNob3VsZCBiZSBwbGFjZWQgYXQgYSBkZXB0aCB0aGF0IGVuc3VyZXMgaGlnaCBkaXNzb2x2ZWQgb3h5Z2VuIGNvbmNlbnRyYXRpb25zIHRvIGF2b2lkIHRoZSBzeXN0ZW0gdG8gYmUgbGltaXRlZC4gSG93ZXZlciwgdG9vIGhpZ2ggZWxlY3RyaWNpdHkgZ2VuZXJhdGlvbiB3YXMgZm91bmQgY291bnRlcnByb2R1Y3RpdmUgZm9yIHJlbWVkaWF0aW9uIHB1cnBvc2VzIGJlY2F1c2UgdGhpcyB3b3VsZCBpbmNyZWFzZSBzZWRpbWVudCB0dXJub3ZlciByYXRlLCByZWxlYXNpbmcgcG9sbHV0YW50cyBzdG9yZWQgaW4gdGhlIHNlZGltZW50cy4gUCBiaW5kaW5nIHdhcyBub3QgZm91bmQgdG8gcmVxdWlyZSBoaWdoIHR1cm5vdmVyIG9mIHNlZGltZW50IG9yZ2FuaWMgbWF0dGVyLiBUaGUgdHVybm92ZXIgY2FuIGJlIGNvbnRyb2xsZWQgYnkgY2hhbmdpbmcgdGhlIGV4dGVybmFsIHJlc2lzdGFuY2UuIEhpZ2ggcmVzaXN0YW5jZSBsaW1pdHMgdGhlIGVsZWN0cmljYWwgcGVyZm9ybWFuY2UsIGxvdyByZXNpc3RhbmNlIGluY3JlYXNlcyBzZWRpbWVudCB0dXJub3ZlciBhbmQgcmlza3MgZGVzdGFiaWxpemluZyB0aGUgc3lzdGVtLiBUbyBhY2hpZXZlIHJlZHVjZWQgY29uY2VudHJhdGlvbnMgb2YgcGhvc3BoYXRlIGluIHRoZSB3YXRlciBjb2x1bW4sIGFub2RlcyBtdXN0IGJlIHBsYWNlZCBpbiB0aGUgc2VkaW1lbnQgc3VyZmFjZSBzaW5jZSBhIGZldyBtaWxsaW1ldHJlcyBvZiBzZWRpbWVudCBtYXkgY29udGFpbiBhbmQgcmVsZWFzZSBlbm91Z2ggUCB0byBzdXN0YWluIGV1dHJvcGhpY2F0aW9uLiBQaG9zcGhhdGUgd2FzIG1lYXN1cmVkIGluIHRoZSB3YXRlciBjb2x1bW4gbmVhciB0aGUgc2VkaW1lbnQsIGluIHN5c3RlbXMgd2l0aCBhbmQgd2l0aG91dCBlbGVjdHJvZGVzLiBQaG9zcGhhdGUgY29uY2VudHJhdGlvbnMgd2VyZSBzdWNjZXNzZnVsbHkgcmVkdWNlZCBieSB1cCB0byA5NSAlIGluIDItY2hhbWJlciBsYWJvcmF0b3J5IHJlYWN0b3JzLCBhbmQgYnkgdXAgdG8gOTggJSBpbiB0aGUgcGlsb3QgaW5zdGFsbGF0aW9uLiBJbiB0aGUgcGlsb3QgZXhwZXJpbWVudCwgcGhvc3BoYXRlIGNvbmNlbnRyYXRpb25zIGFib3ZlIGVsZWN0cm9kZXMgd2VyZSBsb3dlciBpbiBhbGwgbWVhc3VyZW1lbnRzLCBjb21wYXJlZCB0byBjb250cm9scy4gU2VkaW1lbnQgcmVkb3ggcG90ZW50aWFscyBpbmNyZWFzZWQgZnJvbSAtNDAwIG1WIHZzIHN0YW5kYXJkIGh5ZHJvZ2VuIGVsZWN0cm9kZSAoU0hFKSB0byAtMTc1IG1WIHZzIFNIRSwgY29ycmVzcG9uZGluZyB0byBhIG1vdmUgZnJvbSBhIG1ldGhhbm9nZW5ldGljIGRvbWFpbiB0byBzdWxwaHVyIHJlZHVjaW5nLiBBbmFseXNpcyBvZiB0aGUgbWljcm9iaWFsIGNvbW11bml0eSBpbmRpY2F0ZWQgZW5oYW5jZWQgc3VscGh1ciBjeWNsaW5nIHdoZXJlIGVsZWN0cm9kZXMgb3hpZGl6ZSBzdWxwaGlkZSB3aGljaCB0aGUgYmFjdGVyaWEgYWdhaW4gcmVkdWNlIHRvIG9idGFpbiBlbmVyZ3kuIEl0IHdhcyBoeXBvdGhlc2lzZWQgdGhhdCBTTUZDIHByb21vdGUgaGlnaGVyIHJlZG94IHBvdGVudGlhbHMgYnkgZmFjaWxpdGF0aW5nIG94aWRhdGlvbiBvZiBzdWxwaHVyIGFuZCBpcm9uIHNwZWNpZXMsIGluaGliaXRpbmcgSDJTIGZvcm1hdGlvbiBhbmQgRmUgc2NhdmVuZ2luZyBieSBGZVMgZm9ybWF0aW9uLCB0aHVzIGFsbG93aW5nIGZvciBQIGJpbmRpbmcuIEEgbG93ZXIgcmVsYXRpdmUgYWJ1bmRhbmNlIG9mIG1ldGhhbm9nZW5zIG9uIGVsZWN0cm9kZXMgY29tcGFyZWQgdG8gY29udHJvbHMgc3VnZ2VzdHMgc3VwcHJlc3Npb24gb3Igb3V0Y29tcGV0aW5nIG9mIG1ldGhhbm9nZW5zLlF1YW50aWZpY2F0aW9uIG9mIG1ldGhhbmUgcHJvZHVjdGlvbiB3YXMgZG9uZSBvbiB0aGUgYmFzaXMgb2YgYSBDSDQ6Q08yIHJhdGlvIHRvIGFsbG93IGNvbXBhcmFiaWxpdHkgYmV0d2VlbiBzeXN0ZW1zLiBNZXRoYW5lIGVtaXNzaW9uIHdhcyByZWR1Y2VkIGJ5IHVwIHRvIDk5ICUgYW5kIG1haW50YWluZWQgaW5oaWJpdGlvbiBmb3Igc2V2ZXJhbCBtb250aHMgaW4gbGFib3JhdG9yeSBzeXN0ZW1zIHdpdGggZWxlY3Ryb2Rlcy4gSW4gdGhlIHBpbG90IGluc3RhbGxhdGlvbnMgbm8gZ2FzIHNhbXBsZXMgY291bGQgYmUgY29sbGVjdGVkLCBhbmQgbm8gQ08yOkNINCByYXRpb3Mgd2VyZSBvYnRhaW5lZC4gQWVyYXRpb24gaXMgdGhlIGN1cnJlbnQgcmVzdG9yYXRpb24gdGVjaG5pcXVlIHRoYXQgbW9zdCBjbG9zZWx5IHJlc2VtYmxlcyB0aGUgcHJvcG9zZWQgU01GQyB0ZWNobmlxdWUuIEFlcmF0aW9uIGhhcyBwcmV2aW91c2x5IGZhaWxlZCB0byByZWR1Y2UgUCByZWxlYXNlIGluIHNldmVyYWwgc2VwYXJhdGUgaW52ZXN0aWdhdGlvbnMuIFNNRkNzIGFzIGEgcmVtZWRpYXRpb24gdG9vbCB3YXMgZm91bmQgdG8gb2ZmZXIgc2V2ZXJhbCBiZW5lZml0cyBvdmVyIGNvbnZlbnRpb25hbCBtZXRob2RzOiBTTUZDcyBvZmZlcnMgY29udGludW91cyBlZmZlY3QsIHdoaWxlIG90aGVyIHRlY2huaXF1ZXMgcmVxdWlyZSBjb250aW51b3VzIG9yIHJlcGVhdGVkIHRyZWF0bWVudC4gU01GQ3MgY2FuIGJlIGNvbnRyb2xsZWQgaW4gcmVhbC10aW1lIGJ5IGFsdGVyaW5nIHRoZSBleHRlcm5hbCByZXNpc3RvciBhbmQgY2FuIGJlIGVhc2lseSBtb25pdG9yZWQgYnkgbWVhc3VyaW5nIHRoZSBwcm9kdWNlZCBlbGVjdHJpY2FsIG91dHB1dC4gSWYgaW1wbGVtZW50ZWQgYXQgbGFyZ2Ugc2NhbGUgaXQgaXMgZXhwZWN0ZWQgdGhhdCB0aGUgZWxlY3Ryb2RlIGZpZWxkcyB3aWxsIGhlbHAgbWFpbnRhaW4gbG93IHBob3NwaGF0ZSBjb25jZW50cmF0aW9ucyBpbiB0aGUgd2F0ZXIgZm9yIGRlY2FkZXMgd2l0aCBtaW5pbWFsIGludGVydmVudGlvbiwgYW5kIGFmdGVyIHNvbWUgdGltZSwgYmVudGhpYyBwbGFudHMgYXJlIGV4cGVjdGVkIHRvIHJldHVybiwgdGh1cyBtYWludGFpbmluZyB0aGUgYmFsYW5jZSBoZW5jZSBmb3J0aC4iLCJ1cmwiOiJodHRwczovL2xvY2FsLmZvcnNrbmluZ3Nwb3J0YWwuZGsvbG9jYWwvZGtpLWNnaS93cy9jcmlzLWxpbms%2Fc3JjPWR0dSZpZD1kdHUtZDY2ZDMzNzAtMjU2Zi00ODg1LWEwOWUtMzM2YTkyNGY0YmVkJnRpPVNlZGltZW50JTIwbWljcm9iaWFsJTIwZnVlbCUyMGNlbGxzJTIwZm9yJTIwbGFrZSUyMHJlc3RvcmF0aW9uIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2JhY2tlbmQub3JiaXQuZHR1LmRrL3dzL2ZpbGVzLzI0ODA2OTU4OS9UaGVzaXNfb25saW5lX3ZlcnNpb25fS2FybF9IYXh0aGF1c2VuLnBkZiIsImlzc3VlZERhdGUiOiIyMDIxLTAxLTAxIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0wOCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiUmVzZWFyY2ggUG9ydGFsIERlbm1hcmsiLCJpbXBhY3RGYWN0b3IiOiIwLjA0In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6NzZ9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiVGVjaG5pY2FsIFVuaXZlcnNpdHkgb2YgRGVubWFyayJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MCwiY2l0ZWRCeUNvdW50IjoxfX0sInF1b3RlIjoiRXh0ZXJuYWwgaW5wdXRzIGluY3JlYXNlIFAgY29uY2VudHJhdGlvbiBpbiAgdGhlIHdhdGVyLCBjYXVzaW5nIGFsZ2FsIGJsb29tcyB0aGF0IHNoYWRvdyB0aGUgbGFrZSBib3R0b20sIGFuZCBiZW50aGljIG1hYy1yb3BoeXRlcyByZWNlZGUuIFByZWRhdG9yeSBmaXNoIGFyZSB1bmFibGUgdG8gaHVudCwgYW5kIHRoZWlyIGZvb2Qgc291cmNlIGlzICBkaXNydXB0ZWQgc3VjaCBhcyBmaXNoIGZlZWRpbmcgb24gYmVudGhpYyBpbnZlcnRlYnJhdGVzIGFuZCBib3R0b20tZHdlbGxpbmcgYW5pbWFscyBhcmUgc3VmZm9jYXRlZCAoVmFuZGVyIFphbmRlbiBhbmQgVmFkZWJvbmNvZXVyIDIwMDIpLCBzbyB0aGF0IHBsYW5rdGl2b3JvdXMgZmlzaCBzcGVjaWVzIGRvbWluYXRlLiBUaGlzIGluY3JlYXNlcyBwcmVkYXRpb24gb24gem9vIHBsYW5rdG9uICBhbmQgcmVkdWNlcyBncmF6aW5nIG9uIGFsZ2FlLiBBbGdhZSBwcm9saWZlcmF0ZSwgZGllLCBhbmQgc2luayB0byB0aGUgYm90dG9tLCAgY29uc3VtaW5nIG94eWdlbiB3aGVuIGRlZ3JhZGVkLCB3aGljaCBsZWFkcyB0byBveHlnZW4gZGVmaWNpdCBpbiB0aGUgYm90LXRvbSwga2lsbGluZyBmaXNoIChXdXJ0c2JhdWdoIGV0IGFsLiAyMDE5KS4gRmlndXJlIDI6IFZpY2lvdXMgY3ljbGVzIG9mIGV1dHJvcGhpY2F0aW9uLiBMZWZ0OiBJbnRlcmxpbmtlZCBwcm9jZXNzZXMgc3VzdGFpbmluZyBldXRyb3BoaS1jYXRpb24gaW4gYSBsYWtlLiBCbHVlOiBwcm9jZXNzZXMgaW4gd2F0ZXIsIGdyZWVuOiBwcm9jZXNzZXMgaW52b2x2aW5nIG1pY3JvYWxnYWUsIGdyZXk6IHNlZC1pbWVudCByZWxhdGVkIHByb2Nlc3NlcywgeWVsbG93OiBwcm9jZXNzZXMgaW52b2x2aW5nIGZhdW5hLiBSaWdodCwgZGFzaGVkIGxpbmU6IFRoZSBvcmRlciBpbiB3aGljaCBlbGVjdHJvbiBhY2NlcHRvcnMgYXJlIGNvbnN1bWVkIGluIHRoZSBhYnNlbmNlIG9mIE8yIHNob3duIGFzIGNvbmNlbnRyYXRpb24gdmVyc3VzICBkZXB0aCBpbiBzZWRpbWVudCwgYW5kIGF0IHdoaWNoIHBvdGVudGlhbCB0aGlzIG9jY3VycyAoQmVybmVyIDE5ODEsIGJ5IGNvdXJ0ZXN5IG9mIHRoZSBzb2NpZXR5ICBmb3Igc2VkaW1lbnRhcnkgZ2VvbG9neSkuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxNywiYmJveCI6eyJ0b3AiOjc5LjkzMzE1MTI0NTExNzE5LCJsZWZ0IjoxNC4yOTU1MDM2MTYzMzMwMDgsIndpZHRoIjo3MS45Mzg5MzQzMjYxNzE4OCwiaGVpZ2h0IjoyLjA1MzQ3NzI4NzI5MjQ4MDV9fSx7InBhZ2VOdW1iZXIiOjE3LCJiYm94Ijp7InRvcCI6ODEuOTcxMzQzOTk0MTQwNjIsImxlZnQiOjE0LjI5NTUwMzYxNjMzMzAwOCwid2lkdGgiOjcxLjMzMzAwMDE4MzEwNTQ3LCJoZWlnaHQiOjIuMDUzNDc3Mjg3MjkyNDgwNX19LHsicGFnZU51bWJlciI6MTcsImJib3giOnsidG9wIjo4NC4wMjM3OTYwODE1NDI5NywibGVmdCI6MTQuMjk1NTAzNjE2MzMzMDA4LCJ3aWR0aCI6NzEuODEyMDE5MzQ4MTQ0NTMsImhlaWdodCI6Mi4wNTM0NzcyODcyOTI0ODA1fX0seyJwYWdlTnVtYmVyIjoxNywiYmJveCI6eyJ0b3AiOjg2LjA2MTk5NjQ1OTk2MDk0LCJsZWZ0IjoxNC4yOTU1MDM2MTYzMzMwMDgsIndpZHRoIjo3MS45NTczNTkzMTM5NjQ4NCwiaGVpZ2h0IjoyLjA1MzQ3NzI4NzI5MjQ4MDV9fSx7InBhZ2VOdW1iZXIiOjE4LCJiYm94Ijp7InRvcCI6OS45MjY0ODc5MjI2Njg0NTcsImxlZnQiOjE0LjI5NTUwMzYxNjMzMzAwOCwid2lkdGgiOjcxLjk3NDA2MDA1ODU5Mzc1LCJoZWlnaHQiOjEwLjIzNDc3NzQ1MDU2MTUyM319LHsicGFnZU51bWJlciI6MTgsImJib3giOnsidG9wIjo0NC43MzQwNDMxMjEzMzc4OSwibGVmdCI6MTQuMjk1NTAzNjE2MzMzMDA4LCJ3aWR0aCI6NzEuODkzNDMyNjE3MTg3NSwiaGVpZ2h0IjoxMC40MDMyODk3OTQ5MjE4NzV9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI5MyJ9fV0%3D. Projections under RCP 4.5 forecast 70% of tourist lakes unviable by 2050 without restoration, imperiling 2-3 million jobs (Nõges et al., 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRpaW5hIiwiZmFtaWx5IjoiTsO1Z2VzIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hc2ltZSIsImZhbWlseSI6IkphbmF0aWFuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlJlZXQiLCJmYW1pbHkiOiJMYXVnYXN0ZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQZWV0ZXIiLCJmYW1pbHkiOiJOw7VnZXMifX1dLCJpZHMiOnsiaXNzbiI6WyIyMzUxLTk4OTQiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMTYvai5nZWNjby4yMDIwLmUwMTM2OSIsIm1hZyI6IjMxMDU0NDYwOTAiLCJvcGVuYWxleCI6IlczMTA1NDQ2MDkwIiwiZXhhIjoiaHR0cHM6Ly93d3cuc2NpZW5jZWRpcmVjdC5jb20vc2NpZW5jZS9hcnRpY2xlL3BpaS9TMjM1MTk4OTQyMDMwOTEwMCJ9LCJ0aXRsZSI6IlBvc3Qtc292aWV0IGNoYW5nZXMgaW4gbml0cm9nZW4gYW5kIHBob3NwaG9ydXMgc3RvaWNoaW9tZXRyeSBpbiB0d28gbGFyZ2Ugbm9uLXN0cmF0aWZpZWQgbGFrZXMgYW5kIHRoZSBpbXBhY3Qgb24gcGh5dG9wbGFua3RvbiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2ouZ2VjY28uMjAyMC5lMDEzNjkiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2ouZ2VjY28uMjAyMC5lMDEzNjkiLCJpc3N1ZWREYXRlIjoiMjAyMC0xMS0yMCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTEiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6Ikdsb2JhbCBFY29sb2d5IGFuZCBDb25zZXJ2YXRpb24iLCJ2b2x1bWUiOjI0LCJpbXBhY3RGYWN0b3IiOiIzLjU5In0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkVsc2V2aWVyIEJWIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo3MSwiY2l0ZWRCeUNvdW50IjoxNH19LCJxdW90ZSI6IlJhbmRvbSBGb3Jlc3QgYW5hbHlzaXMgaW5kaWNhdGVkIGFuIGltcG9ydGFudCByb2xlIG9mIGxpZ2h0IGxpbWl0YXRpb24gaW4gYm90aCBsYWtlcy4gQ29uc3RhbnRseSBsb3dlciBsZXZlbHMgb2YgTiBpbiB0aGUgZGVlcGVyIExha2UgUGVpcHNpIGZhdm91cmVkIE4yLWZpeGluZyBzcGVjaWVzLCB3aGljaCwgYXMgYSBwYXJhZG94LCBiZWNhbWUgUC1saW1pdGVkLiBBcyBjbGltYXRlIHdhcm1pbmcgcmVpbmZvcmNlcyBldXRyb3BoaWNhdGlvbiBwaGVub21lbmEgaW4gbGFrZXMgYnkgaW5jcmVhc2luZyBpbnRlcm5hbCBudXRyaWVudCBsb2FkaW5nIGFuZCBmYXZvdXJpbmcgYmxvb20tZm9ybWluZyBjeWFub2JhY3RlcmlhLCBtb3JlIHN0cmluZ2VudCBtZWFzdXJlcyB3b3VsZCBiZSBuZWVkZWQgdG8gZnVydGhlciBsaW1pdCBudXRyaWVudCBsb2FkcyAoZXNwZWNpYWxseSB0aGF0IG9mIFApIHRvIGxha2VzIHRocm91Z2ggaW1wcm92ZWQgd2FzdGV3YXRlciB0cmVhdG1lbnQgYW5kIGluY3JlYXNlZCBlZmZpY2llbmN5IG9mIGZlcnRpbGlzZXIgYXBwbGljYXRpb24uIExvbmctdGVybSBjaGFuZ2VzIGluIG51dHJpZW50IGxvYWRpbmcgYW5kIHN0b2ljaGlvbWV0cnkgaW4gd2F0ZXIgYm9kaWVzIHJlZmxlY3QgY2hhbmdlcyBpbiBhZ3JpY3VsdHVyYWwgcHJhY3RpY2VzIGFuZCB3YXN0ZXdhdGVyIHRyZWF0bWVudC4ifV0%3D.

These sectoral shocks ripple into broader socio-economic vulnerabilities, disproportionately affecting marginalized communities. The principal socio-economic impacts of harmful algal blooms on local communities are summarized in Table 4. Women, integral to post-harvest processing and lake-edge farming, endure heightened unpaid labor amid yield drops(Nair & Nayak, 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hdnlhIFZpa3JhbWFuIiwiZmFtaWx5IjoiTmFpciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQcmF0ZWVwIEt1bWFyIiwiZmFtaWx5IjoiTmF5YWsifX1dLCJpZHMiOnsiaXNzbiI6WyIyMjk2LTc3NDUiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzODkvZm1hcnMuMjAyMy4xMDg3Mjk2Iiwib3BlbmFsZXgiOiJXNDM4MjcyMzEzMyJ9LCJ0aXRsZSI6IlVuY292ZXJpbmcgd2F0ZXIgcXVhbGl0eSBhbmQgZXZhbHVhdGluZyB2dWxuZXJhYmlsaXRpZXMgb2Ygc21hbGwtc2NhbGUgZmlzaGVyaWVzIGluIENoaWxpa2EgTGFnb29uLCBJbmRpYSIsImFic3RyYWN0IjoiU21hbGwtc2NhbGUgZmlzaGVyaWVzIChTU0ZzKSBwbGF5IGEgc3Ryb25nIHJvbGUgaW4gc3VzdGFpbmluZyBtaWxsaW9ucyBvZiBsaXZlbGlob29kcywgZm9vZCBzZWN1cml0eSwgbnV0cml0aW9uLCBhbmQgaW5jb21lIGdsb2JhbGx5IGJ1dCB0aGUgZmlzaGVycyBlbmdhZ2VkIGluIHRoaXMgc2VjdG9yIHNpbXVsdGFuZW91c2x5IGV4cGVyaWVuY2UgaGlnaCBsZXZlbHMgb2YgdnVsbmVyYWJpbGl0eSBhbmQgcHJvY2Vzc2VzIG9mIG1hcmdpbmFsaXNhdGlvbi4gU2V2ZXJhbCBmYWN0b3JzIGFyZSBhdHRyaWJ1dGVkIHRvIHRoZSBtdWx0aWRpbWVuc2lvbmFsIHZ1bG5lcmFiaWxpdGllcyB0aGUgc21hbGwtc2NhbGUgZmlzaGVycyBleHBlcmllbmNlLCBzcGFubmluZyBib3RoIG5hdHVyYWwgKGUuZy4sIG5hdHVyYWwgZGlzYXN0ZXJzLCBlY29zeXN0ZW0gY2hhbmdlKSBhbmQgYW50aHJvcG9nZW5pYyAoZS5nLiwgcG9saWN5IGNoYW5nZSwgaHlkcm9sb2dpY2FsIGludGVydmVudGlvbnMsIGFxdWFjdWx0dXJlKSBwcmVzc3VyZXMuIFdoaWxlIHRoZXJlIGlzIG11Y2ggbGl0ZXJhdHVyZSBvbiB2YXJpb3VzIG5hdHVyYWwgYW5kIGh1bWFuIGRyaXZlcnMgb2YgdnVsbmVyYWJpbGl0eSBpbiBzbWFsbC1zY2FsZSBmaXNoZXJ5IGNvbW11bml0aWVzLCBhbiBhYnNlbmNlIG9mIHJlc2VhcmNoIGNvbm5lY3RpbmcgdnVsbmVyYWJpbGl0eSB3aXRoIHdhdGVyIHF1YWxpdHkgaXMgZXZpZGVudC4gRmlzaGVyIGNvbW11bml0aWVzIG9mdGVuIHRhbGsgYWJvdXQgZmlzaCBpbiByZWxhdGlvbiB0byB0aGUgaGVhbHRoIG9mIHRoZWlyIGFxdWF0aWMgaGFiaXRhdHMgd2hlcmVpbiB3YXRlciBxdWFsaXR5IGlzIHNlZW4gYXMgYSBrZXkgcGFyYW1ldGVyLiBUaGUgbGluayBiZXR3ZWVuIGhlYWx0aHkgZmlzaCBhbmQgZ29vZCBxdWFsaXR5IHdhdGVyIGhhcyBzaWduaWZpY2FudCBpbXBsaWNhdGlvbnMgZm9yIHN0cm9uZyBhbmQgdmlhYmxlIGZpc2hpbmcgY29tbXVuaXRpZXMuIFRoaXMgcGFwZXIgZXhhbWluZXMgdGhlc2UgbGlua3MgZnVydGhlciBieSBmb2N1c2luZyBvbiB0aGUgbmF0dXJlIG9mIHZ1bG5lcmFiaWxpdGllcyBjYXVzZWQgYnkgd2F0ZXIgcXVhbGl0eSBjaGFuZ2VzIGluIHRoZSBzbWFsbC1zY2FsZSBmaXNoZXJ5IHN5c3RlbSBvZiBDaGlsaWthIExhZ29vbiBpbiBJbmRpYS4gV2UgdW5kZXJ0YWtlIGRldGFpbGVkIGFuYWx5c2lzIG9mIHRoZSBpbnZhc2l2ZSBzaHJpbXAgYXF1YWN1bHR1cmUgYWN0aXZpdGllcyBhbmQgaHlkcm9sb2dpY2FsIGludGVydmVudGlvbnMgZm9yIG9wZW5pbmcgb2YgYSBsYWdvb24gaW5sZXQgd2l0aCB0aGUgQmF5IG9mIEJlbmdhbCBhcyB0d28gZG9taW5hbnQgZHJpdmVycyBhZHZlcnNlbHkgaW1wYWN0aW5nIHdhdGVyIHF1YWxpdHkgYW5kIGluY3JlYXNpbmcgdnVsbmVyYWJpbGl0aWVzIG9mIHRoZSBlbnRpcmUgc21hbGwtc2NhbGUgZmlzaGVyaWVzIHNvY2lhbC1lY29sb2dpY2FsIHN5c3RlbS4gT3VyIGFuYWx5c2lzIHN1Z2dlc3RzIHRoYXQgdGhlcmUgYXJlIHN0cm9uZyBpbnRlcmNvbm5lY3Rpb25zIGJldHdlZW4gY2hhbmdlcyBpbiB3YXRlciBxdWFsaXR5IGFuZCB0aGUgbGV2ZWxzIG9mIHZ1bG5lcmFiaWxpdGllcyBpbiB0aGUgU1NGcyBvZiBDaGlsaWthIExhZ29vbi4gUG9sbHV0YW50cyBzdWNoIGFzIHBlc3RpY2lkZXMsIGFuZCBvcmdhbmljIGNvbXBvdW5kcyBhY2N1bXVsYXRlIGluIGZpc2ggdGlzc3VlcyBhbmQgYWZmZWN0IHRoZWlyIGdyb3d0aCwgcmVwcm9kdWN0aW9uLCBhbmQgb3ZlcmFsbCBoZWFsdGguIFRoaXMgbGVkIHRvIGRlY2xpbmVzIGluIGZpc2ggcG9wdWxhdGlvbnMsIG1ha2luZyBpdCBtb3JlIGRpZmZpY3VsdCBmb3IgZmlzaGVycyB0byBtYWtlIGEgbGl2aW5nLiBJbiBhZGRpdGlvbiB0byBkaXJlY3QgaW1wYWN0cyBvbiBmaXNoIHBvcHVsYXRpb25zLCBwb29yIHdhdGVyIHF1YWxpdHkgYWxzbyBoYXMgaW5kaXJlY3QgZWZmZWN0cyBvbiB0aGUgc29jaWFsIGFuZCBlY29ub21pYyB2dWxuZXJhYmlsaXR5IG9mIFNTRnMuIEZvciBleGFtcGxlLCBjb250YW1pbmF0aW9uIG9mIHdhdGVyIHNvdXJjZXMgbGVkIHRvIHRoZSByZWR1Y2VkIG51bWJlciBvZiBmaXNoIHNwZWNpZXMgcmVkdWNpbmcgdGhlIGFtb3VudCBvZiB0aW1lIGZpc2hlcnMgY2FuIHNwZW5kIG9uIGZpc2hpbmcgYWN0aXZpdGllcy4gVGhpcyBhbHNvIGFmZmVjdGVkIHRoZSBtYXJrZXRhYmlsaXR5IG9mIGZpc2ggcHJvZHVjdHMsIHJlZHVjZWQgaW5jb21lIGFuZCBpbmNyZWFzZWQgcG92ZXJ0eS4gVG8gZnVsbHkgdW5kZXJzdGFuZCB0aGUgaW50ZXJjb25uZWN0aW9ucyBiZXR3ZWVuIHdhdGVyIHF1YWxpdHkgYW5kIHZ1bG5lcmFiaWxpdHkgaW4gU1NGcyBpbiBDaGlsaWthIExhZ29vbiwgaXQgaXMgaW1wb3J0YW50IHRvIGNvbnNpZGVyIGJvdGggZW52aXJvbm1lbnRhbCBhbmQgc29jaWFsIGZhY3RvcnMsIGFzIHdlbGwgYXMgdGhlIGNvbXBsZXggZmVlZGJhY2sgbG9vcHMgYmV0d2VlbiB0aGVzZSBmYWN0b3JzLiBUaGUgc3R1ZHkgaGVscHMgaW4gYnJpZGdpbmcgYSBjcnVjaWFsIGdhcCBpbiBvdXIgdW5kZXJzdGFuZGluZyBvZiB0aGUgcm9sZSBvZiB3YXRlciBxdWFsaXR5IGluIHZ1bG5lcmFiaWxpdHkgYW5hbHlzaXMgd2l0aGluIHJlc291cmNlIGRlcGVuZGVudCBjb21tdW5pdGllcy4gV2UgY29uY2x1ZGUgd2l0aCBrZXkgaW5zaWdodHMgb24gcG9zc2libGUgY29waW5nIHJlc3BvbnNlcyBhbmQgYWRhcHRpdmUgY2FwYWNpdHkgbmVjZXNzYXJ5IGZvciB0aGUgc21hbGwtc2NhbGUgZmlzaGVyaWVzIGNvbW11bml0aWVzIHRvIHRyYW5zaXRpb24gdG93YXJkIHZpYWJpbGl0eS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMzM4OS9mbWFycy4yMDIzLjEwODcyOTYiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vd3d3LmZyb250aWVyc2luLm9yZy9hcnRpY2xlcy8xMC4zMzg5L2ZtYXJzLjIwMjMuMTA4NzI5Ni9wZGYiLCJpc3N1ZWREYXRlIjoiMjAyMy0wNi0yOSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZyb250aWVycyBpbiBNYXJpbmUgU2NpZW5jZSIsInZvbHVtZSI6MTAsImltcGFjdEZhY3RvciI6IjQuMjgifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRnJvbnRpZXJzIE1lZGlhIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo1MSwiY2l0ZWRCeUNvdW50Ijo1fX0sInF1b3RlIjoiV2F0ZXIgcXVhbGl0eSBvZiBDaGlsaWthIExhZ29vbiBoYXMgZGVncmFkZWQgZHVlIHRvIGFudGhyb3BvZ2VuaWMgaW50ZXJ2ZW50aW9ucyBhbmQgbmF0dXJhbCBmb3JjZXMgb2YgY2hhbmdlLiBWYXJpb3VzIGFjdGl2aXRpZXMgaW4gdGhlIENoaWxpa2EgTGFnb29uLCBzdWNoIGFzIGluZHVzdHJpYWwgd2FzdGV3YXRlciBkaXNwb3NhbCwgc2V3YWdlIGR1bXBpbmcsIGFxdWFjdWx0dXJlLCBoeWRyb2xvZ2ljYWwgaW50ZXJ2ZW50aW9ucywgYW5kIGN5Y2xvbmVzLCBhcmUgcHJvZHVjaW5nIHNhbGluaXR5IGNoYW5nZXMsIHNlZGltZW50IGRlcG9zaXRpb24sIG51dHJpZW50IGVucmljaG1lbnQsIGV1dHJvcGhpY2F0aW9uLCBhbmQgZGVhZCB6b25lcywgYXMgc3RhdGVkLiBGaXNoIHN1cnZpdmFsIGlzIGF0IHJpc2sgYXMgYSByZXN1bHQgb2YgdGhlIHNpZ25pZmljYW50IHJlZHVjdGlvbiBpbiBlbnZpcm9ubWVudGFsIGNpcmN1bXN0YW5jZXMsIHdoaWNoIGRpbWluaXNoZXMgZmlzaGVybWVu4oCZcyBpbmNvbWUgYW5kIHB1c2hlcyB0aGVtIG91dCBvZiBzb2NpZXR5LiBUaGVzZSBjYXVzZXMgYWZmZWN0aW5nIHRoZSB3YXRlciBxdWFsaXR5IG9mIHRoZSBMYWdvb24gZGVwaWN0IG1hbnkgZmFjZXMgb2YgdnVsbmVyYWJpbGl0eSBpbiBTU0YgY29tbXVuaXRpZXMuIEFzIHRoZSBDaGlsaWthIExhZ29vbiBlbnZpcm9ubWVudCBpcyBleHBlY3RlZCB0byB1bmRlcmdvIGluY3JlYXNpbmdseSBkcmFzdGljIGNoYW5nZXMgKHN1Y2ggYXMgY2F0Y2ggY29tcG9zaXRpb24sIGNhdGNoIGNhcGFjaXR5LCBiaW9kaXZlcnNpdHkgdmFyaWF0aW9ucywgd2F0ZXIgcXVhbGl0eSwgYW5kIGZpc2hlcnkgcmV2ZW51ZXMpLCBpbnRlcm5hdGlvbmFsIHBvbGljeSBoYXMgYmVjb21lIG1vcmUgaW50ZXJlc3RlZCBpbiBhZGFwdGF0aW9uIG1lYXN1cmVzLiBHaXZlbiBob3cgZGl2ZXJzZSBTU0YgZWNvc3lzdGVtcyBhcmUgZXhwZWN0ZWQgdG8gYmUgYmV0d2VlbiBjb250aW5lbnRzLCB0aGVyZSBpcyBubyDigJxvbmUgc2l6ZSBmaXRzIGFsbOKAnSBhcHByb2FjaC4gVGhlIHBhcGVyIG91dGxpbmVkIHNldmVyYWwgY29waW5nIGFuZCBhZGFwdGF0aW9uIG1lY2hhbmlzbXMgdG8gaW1wcm92ZSB3YXRlciBxdWFsaXR5IGFuZCBjb250aW51ZSBmaXNoaW5nIGluIGEgc3VzdGFpbmFibGUgd2F5LiBBZGFwdGF0aW9uIHN0cmF0ZWdpZXMgY2FuIG9jY2FzaW9uYWxseSBoYXZlIHVuZm9yZXNlZW4gY29uc2VxdWVuY2VzIGZvciBmaXNoZXJpZXMgYW5kIGZpc2hpbmcgY29tbXVuaXRpZXMuIEFueSB1bmV4cGVjdGVkIGNvbnNlcXVlbmNlcyBvZiBhZGFwdGF0aW9uIGNhbiBiZSBtaXRpZ2F0ZWQgdGhyb3VnaCBsb25nLXRlcm0gcGxhbm5pbmcgYW5kIGlkZW50aWZpY2F0aW9uIG9mIGZ1dHVyZSBvcHRpb25zIHRocm91Z2ggc2NlbmFyaW8gYW5hbHlzaXMuIFRoZSBmaW5kaW5ncyBvZiB0aGlzIHBhcGVyIGNhbiBiZSB1dGlsaXplZCB0byBkZXZlbG9wIHBvbGljaWVzIHRvIHByb3RlY3Qgd2F0ZXIgcXVhbGl0eSBhbmQgU1NGcyBpbiBhIHZhcmlldHkgb2YgY29hc3RhbCBsYWdvb24gc2V0dGluZ3MuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjIyLjI4NTAzNjA4NzAzNjEzMywibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ5MzA1MzQzNjI3OTMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjIzLjc3MzUxOTUxNTk5MTIxLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDgzMzY0MTA1MjI0NjEsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjI1LjI1NTU4NDcxNjc5Njg3NSwibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ4OTU1OTE3MzU4Mzk4NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjYuNzQ0MDY4MTQ1NzUxOTUzLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNTA2ODA1NDE5OTIxODc1LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyOC4yMjYxMzMzNDY1NTc2MTcsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40OTA0NjMyNTY4MzU5NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjkuNzE0NjE2Nzc1NTEyNjk1LCJsZWZ0Ijo4LjMzMTU5MjU1OTgxNDQ1Mywid2lkdGgiOjQwLjQ5NDA5ODY2MzMzMDA4LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozMS4xOTY2ODM4ODM2NjY5OTIsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40OTQzOTIzOTUwMTk1MywiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MzIuNjg1MTY1NDA1MjczNDQsImxlZnQiOjguMzMxNTk2Mzc0NTExNzE5LCJ3aWR0aCI6NDAuNDg3NDQ1ODMxMjk4ODMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM0LjE2NzIzMjUxMzQyNzczNCwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40OTc3NDU1MTM5MTYwMTYsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM1LjY1NTcxNTk0MjM4MjgxLCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ5ODYzODE1MzA3NjE3LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozNy4xMzc3ODMwNTA1MzcxMSwibGVmdCI6MTAuNjg1MTg2Mzg2MTA4Mzk4LCJ3aWR0aCI6MzguMTQxMTA1NjUxODU1NDcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM4LjYyNjI2NjQ3OTQ5MjE5LCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4NzE1MjA5OTYwOTM3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDAuMTA4MzMzNTg3NjQ2NDg0LCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4OTg1NjcxOTk3MDcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQxLjU5NjgxMzIwMTkwNDMsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDg4NjY2NTM0NDIzODMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQzLjA3ODg4MDMxMDA1ODU5NCwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40ODE4NjExMTQ1MDE5NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDQuNTY3MzYzNzM5MDEzNjcsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDgzMDU4OTI5NDQzMzYsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQ2LjA0OTQzMDg0NzE2Nzk3LCJsZWZ0Ijo4LjMzMzEwODkwMTk3NzUzOSwid2lkdGgiOjQwLjUwMDc0NzY4MDY2NDA2LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo0Ny41Mzc5MTQyNzYxMjMwNSwibGVmdCI6OC4zMzMxMDg5MDE5Nzc1MzksIndpZHRoIjo0MC40OTU2MDU0Njg3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDkuMDE5OTgxMzg0Mjc3MzQ0LCJsZWZ0Ijo4LjMzMzEwNjA0MDk1NDU5LCJ3aWR0aCI6NDAuNTA2ODE2ODY0MDEzNjcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUwLjUwODQ2NDgxMzIzMjQyLCJsZWZ0Ijo4LjMzMzEwNjA0MDk1NDU5LCJ3aWR0aCI6NDAuNDg4NTA2MzE3MTM4NjcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUxLjk5MDUyODEwNjY4OTQ1LCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDk3NzI2NDQwNDI5NjksImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUzLjQ3OTAxMTUzNTY0NDUzLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6MzMuMTU5NDQ2NzE2MzA4NTk0LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo1NC45NjEwNzg2NDM3OTg4MywibGVmdCI6MTAuNjgzNjcwMDQzOTQ1MzEyLCJ3aWR0aCI6MzguMTM2NTk2Njc5Njg3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NTYuNDQ5NTYyMDcyNzUzOTA2LCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDg3MTQ0NDcwMjE0ODQ0LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEzIn19XQ%3D%3D. Indigenous groups around Chilika report cultural losses from sacred fisheries' decline, intertwining economic with identity erosion (Jilbert et al., 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRvbSIsImZhbWlseSI6IkppbGJlcnQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUmFvdWzigJBNYXJpZSIsImZhbWlseSI6IkNvdXR1cmUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQnJpYW4iLCJmYW1pbHkiOiJIdXNlciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLYWxldmkiLCJmYW1pbHkiOiJTYWxvbmVuIn19XSwiaWRzIjp7Imlzc24iOlsiMDAxOC04MTU4IiwiMTU3My01MTE3Il0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDA3L3MxMDc1MC0wMjAtMDQ0NTcteCIsIm1hZyI6IjMxMDA0NTkwNTAiLCJvcGVuYWxleCI6IlczMTAwNDU5MDUwIn0sInRpdGxlIjoiUHJlZmFjZTogUmVzdG9yYXRpb24gb2YgZXV0cm9waGljIGxha2VzOiBjdXJyZW50IHByYWN0aWNlcyBhbmQgZnV0dXJlIGNoYWxsZW5nZXMiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAwNy9zMTA3NTAtMDIwLTA0NDU3LXgiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vbGluay5zcHJpbmdlci5jb20vY29udGVudC9wZGYvMTAuMTAwNy9zMTA3NTAtMDIwLTA0NDU3LXgucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjAtMTEtMTYiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA3IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJIeWRyb2Jpb2xvZ2lhIiwiaXNzdWUiOjIxLCJ2b2x1bWUiOjg0NywiaW1wYWN0RmFjdG9yIjoiMy4wMCJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjQzNDMsImxhc3RQYWdlIjo0MzU3fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlNwcmluZ2VyIFNjaWVuY2UrQnVzaW5lc3MgTWVkaWEifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjk1LCJjaXRlZEJ5Q291bnQiOjU4fX0sInF1b3RlIjoiU2ltaWxhciBkZWdyYWRhdGlvbiBvZiBmcmVzaHdhdGVyIGVjb3N5c3RlbXMgaXMgb2JzZXJ2ZWQgd29ybGR3aWRlLCB3aXRoIHRoZSB0aW1lbGluZSBvZiBodW1hbiBwcmVzc3VyZSBkaWN0YXRlZCBieSB0cmVuZHMgaW4gdXJiYW5pemF0aW9uIGFuZCBpbnRlbnNpdmUgYWdyaWN1bHR1cmUgaW4gZGlmZmVyZW50IGdlb2dyYXBoaWMgcmVnaW9ucyAoZS5nLiwgVmVubmVtbyBldCBhbC4sIDIwMDkpLiBUaGUgcHJpbmNpcGFsIGRyaXZlciBvZiBlY29zeXN0ZW0gc3RyZXNzIGluIG1vc3QgYXF1YXRpYyBzeXN0ZW1zIGlzIGV1dHJvcGhpY2F0aW9uLCB0aGUgZXhjZXNzIGdyb3d0aCBvZiBhbGdhZSBkcml2ZW4gYnkgYW50aHJvcG9nZW5pYyBpbnB1dHMgb2YgdGhlIG1hY3JvbnV0cmllbnRzIG5pdHJvZ2VuIChOKSBhbmQgcGhvc3Bob3J1cyAoUCkgKFNtaXRoLCAyMDAzOyBCaXJrIGV0IGFsLiwgMjAyMCkuIE51dHJpZW50cyBhcmUgZW1pdHRlZCBmcm9tIGJvdGggcG9pbnQgYW5kIGRpZmZ1c2Ugc291cmNlcyBpbiB0aGUgaHVtYW4gZW52aXJvbm1lbnQgYW5kIGVudGVyIGFxdWF0aWMgc3lzdGVtcyB2aWEgc3VyZmFjZSBydW5vZmYsIGFzIHdlbGwgYXMgZ3JvdW5kLXdhdGVyIGFuZCBhdG1vc3BoZXJpYyBkZXBvc2l0aW9uIChQYWVybCwgMTk5NykuIEV1dHJvcGhpY2F0aW9uIG9mdGVuIGxlYWRzIHRvIHRoZSBkb21pbmFuY2Ugb2YgcHJpbWFyeSBwcm9kdWN0aW9uIGJ5IGN5YW5vYmFjdGVyaWEsIGluY2x1ZGluZyB0b3hpYyBnZW5lcmEgc3VjaCBhcyBEb2xpY2hvc3Blcm11bSwgQXBoYW5pem9tZW5vbiwgYW5kIE1pY3JvY3lzdGlzIChIdWlzbWFuIGV0IGFsLiwgMjAxOCksIGFzIHdlbGwgYXMgb3h5Z2VuIGRlcGxldGlvbiBhbmQgaGFiaXRhdCBkZWdyYWRhdGlvbiBpbiBsYWtlcyAoQXJlbmQgZXQgYWwuLCAyMDExKS4gQmVjYXVzZSBsYWtlcyBwcm92aWRlIGEgcmFuZ2Ugb2YgY3J1Y2lhbCBlY29zeXN0ZW0gc2VydmljZXMgKFdpbHNvbiAmIENhcnBlbnRlciwgMTk5OSksIHRoZXJlIGlzIGEgZ3Jvd2luZyBuZWVkIGZvciBtYW5hZ2VtZW50IGFwcHJvYWNoZXMgdGhhdCByZXN0b3JlIHdhdGVyIHF1YWxpdHkgaW4gZnJlc2h3YXRlciBzeXN0ZW1zLiBXaXRoaW4gdGhlIGZyYW1ld29yayBvZiBVbml0ZWQgTmF0aW9ucyBTdXN0YWluYWJsZSBEZXZlbG9wLW1lbnQgR29hbCA2IChDbGVhbiBXYXRlciBhbmQgU2FuaXRhdGlvbiksIHNldmVyYWwgZGlzdGluY3QgdGFyZ2V0cyByZWxhdGUgdG8gdGhlIGltcHJvdmVtZW50IG9mIHdhdGVyIHF1YWxpdHkgYW5kIHByb3RlY3Rpb24gb2YgYXF1YXRpYyBlY29zeXN0ZW1zIGJ5IDIwMzAgKFVuaXRlZCBOYXRpb25zLCAyMDE1KS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyOC4yOTg4NjQzNjQ2MjQwMjMsImxlZnQiOjguNTQ5MjA1NzgwMDI5Mjk3LCJ3aWR0aCI6MzkuMzcxNDk0MjkzMjEyODksImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MjkuOTkxMTM0NjQzNTU0Njg4LCJsZWZ0Ijo4LjU0OTIwNTc4MDAyOTI5Nywid2lkdGgiOjM5LjM2NTExOTkzNDA4MjAzLCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMxLjY4MzQwNjgyOTgzMzk4NCwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNjk2NjMyMzg1MjUzOSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozMy4zNjc5NzMzMjc2MzY3MiwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNzU2NzkwMTYxMTMyOCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNS4wNjAyNDkzMjg2MTMyOCwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNzA5NjQwNTAyOTI5NywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNi43NTI1MTc3MDAxOTUzMSwibGVmdCI6OC41NDkyMDg2NDEwNTIyNDYsIndpZHRoIjozOS4zNzY5NjA3NTQzOTQ1MywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC40MzcwODgwMTI2OTUzMSwibGVmdCI6OC41NDkyMDg2NDEwNTIyNDYsIndpZHRoIjozOS4zNjY1Njk1MTkwNDI5NywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0MC4xMjkzNTYzODQyNzczNDQsImxlZnQiOjguNTQ5MjA4NjQxMDUyMjQ2LCJ3aWR0aCI6MzkuMzY0Mzc5ODgyODEyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0MS44MjE2MjQ3NTU4NTkzNzUsImxlZnQiOjguNTQ5MjA4NjQxMDUyMjQ2LCJ3aWR0aCI6MzkuMzY1NjczMDY1MTg1NTUsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDMuNTA2MTk1MDY4MzU5Mzc1LCJsZWZ0Ijo4LjU0OTM4OTgzOTE3MjM2Mywid2lkdGgiOjM5LjM3MDM5MTg0NTcwMzEyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0NS4xOTg0NjM0Mzk5NDE0MDYsImxlZnQiOjguNTQ5NTcyOTQ0NjQxMTEzLCJ3aWR0aCI6MzkuMzY1ODQ0NzI2NTYyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ni44OTA3MzU2MjYyMjA3LCJsZWZ0Ijo4LjU0OTU3Mjk0NDY0MTExMywid2lkdGgiOjM5LjM3Mjk1NTMyMjI2NTYyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0OC41NzUzMDIxMjQwMjM0NCwibGVmdCI6OC41NDk1NzI5NDQ2NDExMTMsIndpZHRoIjozOS4zNzI0MjEyNjQ2NDg0NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1MC4yNjc1NzgxMjUsImxlZnQiOjguNTQ5NTc2NzU5MzM4Mzc5LCJ3aWR0aCI6MzkuMzcxMTI0MjY3NTc4MTI1LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjUxLjk1OTg0MjY4MTg4NDc2NiwibGVmdCI6OC41NDk1NzY3NTkzMzgzNzksIndpZHRoIjozOS4zNjc4MzU5OTg1MzUxNTYsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTMuNTc0MTE5NTY3ODcxMDk0LCJsZWZ0Ijo4LjU0OTU3Njc1OTMzODM3OSwid2lkdGgiOjM5LjM3MDc3NzEzMDEyNjk1LCJoZWlnaHQiOjEuMzM2ODk1NTg1MDYwMTE5Nn19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjU1LjI2NjM5MTc1NDE1MDM5LCJsZWZ0Ijo4LjU0OTc2MDgxODQ4MTQ0NSwid2lkdGgiOjM5LjM2NjU4MDk2MzEzNDc2NiwiaGVpZ2h0IjoxLjMzNjg5NTU4NTA2MDExOTZ9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1Ny4wMjg5NTM1NTIyNDYwOTQsImxlZnQiOjguNTQ5OTQ0ODc3NjI0NTEyLCJ3aWR0aCI6MzkuMzYyMDI2MjE0NTk5NjEsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTguNzEzNTIzODY0NzQ2MDk0LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjkuMjk5OTgyMDcwOTIyODUyLCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjYwLjQwNTc5MjIzNjMyODEyNSwibGVmdCI6MTAuNjIyNDU1NTk2OTIzODI4LCJ3aWR0aCI6MzcuMjg3ODY4NDk5NzU1ODYsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NjIuMDk4MDYwNjA3OTEwMTU2LCJsZWZ0Ijo4LjU0OTk0MzkyMzk1MDE5NSwid2lkdGgiOjM5LjM3MzMyNTM0NzkwMDM5LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjYzLjc4MjYzMDkyMDQxMDE1NiwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzMxMjY5ODM2NDI1OCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2NS40NzQ5MDY5MjEzODY3MiwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNjUxMTk5MzQwODIwMywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2Ny4xNjcxNzUyOTI5Njg3NSwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNjk0ODAxMzMwNTY2NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2OC43ODE0NDgzNjQyNTc4MSwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzczMzg0MDk0MjM4MywiaGVpZ2h0IjoxLjMzNjg5MTUzMTk0NDI3NX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjcwLjU0NDAxMzk3NzA1MDc4LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjM5LjM2OTMwODQ3MTY3OTY5LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjcyLjIzNjI4OTk3ODAyNzM0LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjM5LjM3MjAzMjE2NTUyNzM0NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo3My45MjA4NDUwMzE3MzgyOCwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzEzMDczNzMwNDY4NzUsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNDM0NCJ9fV0%3D. Health expenditures from cyanotoxin exposures strain household budgets, with hospital admissions for dermatitis and hepatotoxicity surging 30-40% post-blooms (Gobler et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkNocmlzdG9waGVyIEouIiwiZmFtaWx5IjoiR29ibGVyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlJ1dGggVy4iLCJmYW1pbHkiOiJEcmlua3dhdGVyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkEuIFcuIiwiZmFtaWx5IjoiQW50aG9ueSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJKZW5uaWZlciBBLiIsImZhbWlseSI6IkdvbGVza2kifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW5uIE1hcmllIEUuIiwiZmFtaWx5IjoiRmFtdWxhcm8tUGVjb3JhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1pY2hhZWwgQi4iLCJmYW1pbHkiOiJXYWxsYWNlIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5vcmEgUi4gVy4iLCJmYW1pbHkiOiJTdHJhcXVhZGluZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJSb25vam95IiwiZmFtaWx5IjoiSGVtIn19XSwiaWRzIjp7Imlzc24iOlsiMTY2NC0zMDJYIl0sImlzYm4iOltdLCJkb2kiOiIxMC4zMzg5L2ZtaWNiLjIwMjQuMTQ2NDY4NiIsInBtaWQiOiIzOTU2ODk5NyIsIm9wZW5hbGV4IjoiVzQ0MDQxMjA5NTIiLCJleGEiOiJodHRwczovL3d3dy5mcm9udGllcnNpbi5vcmcvam91cm5hbHMvbWljcm9iaW9sb2d5L2FydGljbGVzLzEwLjMzODkvZm1pY2IuMjAyNC4xNDY0Njg2L2Z1bGwifSwidGl0bGUiOiJTZXdhZ2UtYW5kIGZlcnRpbGl6ZXItZGVyaXZlZCBudXRyaWVudHMgYWx0ZXIgdGhlIGludGVuc2l0eSwgZGl2ZXJzaXR5LCBhbmQgdG94aWNpdHkgb2YgaGFybWZ1bCBjeWFub2JhY3RlcmlhbCBibG9vbXMgaW4gZXV0cm9waGljIGxha2VzIiwiYWJzdHJhY3QiOiJDeWFub2JhY3RlcmlhbCBoYXJtZnVsIGFsZ2FsIGJsb29tcyAoQ0hBQnMpIGFyZSBwcm9tb3RlZCBieSBleGNlc3NpdmUgbnV0cmllbnQgbG9hZGluZyBhbmQsIHdoaWxlIGZlcnRpbGl6ZXJzIGFuZCBzZXdhZ2UgYXJlIHRoZSBtb3N0IHByZXZhbGVudCBleHRlcm5hbCBudXRyaWVudCBzb3VyY2VzIGluIG1vc3Qgd2F0ZXJzaGVkcywgdGhlIGRpZmZlcmVudGlhbCBlZmZlY3RzIG9mIHRoZXNlIG51dHJpZW50IHNvdXJjZXMgb24gQ0hBQnMgYXJlIHVua25vd24uIEhlcmUsIHdlIHRyYWNrZWQgQ0hBQnMgYW5kIHBlcmZvcm1lZCBleHBlcmltZW50cyBpbiBmaXZlIGRpc3RpbmN0IGxha2VzIGFjcm9zcyB0aGUgTm9ydGhlcm4gVVMgaW5jbHVkaW5nIExha2UgRXJpZS4gRmVydGlsaXplcnMgd2l0aCBhbW1vbml1bSBhbmQgb3J0aG9waG9zcGhhdGUsIG1lbWJyYW5lICgwLjIgzrxtKS1maWx0ZXJlZCBzZXdhZ2UgKGRvbWluYXRlZCBieSByZWR1Y2VkIGZvcm1zIG9mIG5pdHJvZ2VuKSBzYW5kLWFuZCBtZW1icmFuZS1maWx0ZXJlZCBzZXdhZ2UgKGRvbWluYXRlZCBieSBuaXRyYXRlKSwgYW5kIGFuIGlub3JnYW5pYyBudXRyaWVudCBzb2x1dGlvbiBvZiBhbW1vbml1bSBhbmQgb3J0aG9waG9zcGhhdGUgd2VyZSB1c2VkIGFzIGV4cGVyaW1lbnRhbCBudXRyaWVudCBzb3VyY2VzIGZvciBDSEFCcyBhdCBOLWVxdWl2YWxlbnQsIGVudmlyb25tZW50YWxseSByZWFsaXN0aWMgY29uY2VudHJhdGlvbnMuIFBoeXRvcGxhbmt0b24gY29tbXVuaXRpZXMgd2VyZSBldmFsdWF0ZWQgZmx1b3JvbWV0cmljYWxseSwgbWljcm9zY29waWNhbGx5LCBhbmQgdmlhIGhpZ2ggdGhyb3VnaHB1dCBzZXF1ZW5jaW5nIG9mIHRoZSAxNlMgclJOQSBnZW5lLCBhbmQgbGV2ZWxzIG9mIG1pY3JvY3lzdGluIGFuZCB0aGUgzrQgMTUgTiBjb250ZW50IG9mIHBhcnRpY3VsYXRlIG9yZ2FuaWMgbml0cm9nZW4gKM60UE8gMTUgTikgd2VyZSBxdWFudGlmaWVkLiBGZXJ0aWxpemVyIGFuZCBib3RoIHNvdXJjZXMgb2Ygd2FzdGV3YXRlciBpbmNyZWFzZWQgdGhlIGFidW5kYW5jZSBvZiBjeWFub2JhY3RlcmlhIGluIGFsbCBleHBlcmltZW50cyBhY3Jvc3MgYWxsIGZpdmUgbGFrZXMgKCBwICZhbXA7bHQ7IDAuMDUgZm9yIGFsbCkgd2hlcmVhcyBlZmZlY3RzIG9uIGV1a2FyeW90aWMgcGh5dG9wbGFua3RvbiB3ZXJlIGxpbWl0ZWQuIFNhbmQtZmlsdGVyZWQgc2V3YWdlIGNvbnRhaW5lZCBsZXNzIFAsIG9yZ2FuaWMgbWF0dGVyLCBhbmQgYW1tb25pdW0gYnV0IG1vcmUgbml0cmF0ZSBhbmQgaGFkIGEgMjUlIGxlc3MgcG90ZW50IHN0aW11bGF0b3J5IGVmZmVjdCBvbiBjeWFub2JhY3RlcmlhIHRoYW4gbWVtYnJhbmUtZmlsdGVyZWQgc2V3YWdlLCBzdWdnZXN0aW5nIG5pdHJpZmljYXRpb24gbWF5IHBsYXkgYSByb2xlIGluIHJlZHVjaW5nIENIQUJzLiBGZXJ0aWxpemVyIGluY3JlYXNlZCBtaWNyb2N5c3RpbiBsZXZlbHMgYW5kIGRlY3JlYXNlZCB0aGUgzrRQTyAxNSBOIHdoZXJlYXMgd2FzdGV3YXRlciBpbmNyZWFzZWQgzrRQTyAxNSBOICggcCAmYW1wO2x0OyAwLjA1IGZvciBhbGwpLiBNaWNyb2N5c3RpcyB3YXMgdGhlIGdlbnVzIG1vc3QgY29uc2lzdGVudGx5IHByb21vdGVkIGJ5IG51dHJpZW50IHNvdXJjZXMgKCBwICZhbXA7bHQ7IDAuMDUgaW4gYWxsIGV4cGVyaW1lbnRzKSwgZm9sbG93ZWQgYnkgQ3lhbm9iaXVtICggcCAmYW1wO2x0OyAwLjA1IGluIDUwJSBvZiBleHBlcmltZW50cyksIHdpdGggaW5jcmVhc2VzIGluIE1pY3JvY3lzdGlzIGJpb21hc3MgY29uc2lzdGVudGx5IGVsaWNpdGVkIGJ5IG1lbWJyYW5lLWZpbHRlcmVkIHdhc3Rld2F0ZXIuIENvbGxlY3RpdmVseSwgcmVzdWx0cyBkZW1vbnN0cmF0ZSB0aGF0IGRpZmZlcmluZyB0eXBlcyBvZiBzZXdhZ2UgZGlzY2hhcmdlIGFuZCBmZXJ0aWxpemVycyBjYW4gcHJvbW90ZSBDSEFCIGludGVuc2l0eSBhbmQgdG94aWNpdHksIHdoaWxlIGNvbmN1cnJlbnRseSBhbHRlcmluZyBDSEFCIGRpdmVyc2l0eSBhbmQgzrRQTyAxNSBOLiBXaGlsZSBtZW1icmFuZS1maWx0ZXJlZCBzZXdhZ2UgY29uc2lzdGVudGx5IGZhdm9yZWQgTWljcm9jeXN0aXMgLCB0aGUgZGlzY2hhcmdlIG9mIHNld2FnZSB0aHJvdWdoIHNhbmRzIG11dGVkIGJsb29tIGludGVuc2l0eSBzdWdnZXN0aW5nIHNhbmQtYmVkcyBtYXkgcmVwcmVzZW50IGEgdG9vbCB0byByZW1vdmUga2V5IG51dHJpZW50cyBhbmQgcGFydGlhbGx5IG1pdGlnYXRlIENIQUJzLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzg5L2ZtaWNiLjIwMjQuMTQ2NDY4NiIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cuZnJvbnRpZXJzaW4ub3JnL2pvdXJuYWxzL21pY3JvYmlvbG9neS9hcnRpY2xlcy8xMC4zMzg5L2ZtaWNiLjIwMjQuMTQ2NDY4Ni9wZGYiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMS0wNiIsImFjY2Vzc2VkRGF0ZSI6IjIwMjYtMDIiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZyb250aWVycyBpbiBNaWNyb2Jpb2xvZ3kiLCJ2b2x1bWUiOjE1LCJpbXBhY3RGYWN0b3IiOiIzLjk5In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTQ2NDY4NiwibGFzdFBhZ2UiOjE0NjQ2ODZ9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRnJvbnRpZXJzIE1lZGlhIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo3MywiY2l0ZWRCeUNvdW50Ijo5fX0sInF1b3RlIjoiZGVyaXZlZCBudXRyaWVudHMgYWx0ZXIgdGhlIGludGVuc2l0eSwgZGl2ZXJzaXR5LCBhbmQgdG94aWNpdHkgb2YgaGFybWZ1bCBjeWFub2JhY3RlcmlhbCBibG9vbXMgLi4uIEN5YW5vYmFjdGVyaWFsIGhhcm1mdWwgYWxnYWwgYmxvb21zIChDSEFCcykgYXJlIHByb21vdGVkIGJ5IGV4Y2Vzc2l2ZSBudXRyaWVudCBsb2FkaW5nIGFuZCwgd2hpbGUgZmVydGlsaXplcnMgYW5kIHNld2FnZSBhcmUgdGhlIG1vc3QgcHJldmFsZW50IGV4dGVybmFsIG51dHJpZW50IHNvdXJjZXMgaW4gbW9zdCB3YXRlcnNoZWRzLCB0aGUgZGlmZmVyZW50aWFsIGVmZmVjdHMgb2YgdGhlc2UgbnV0cmllbnQgLi4uIG9yZ2FuaWMgbml0cm9nZW4gKM60UE8xNU4pIHdlcmUgcXVhbnRpZmllZC4gRmVydGlsaXplciBhbmQgYm90aCBzb3VyY2VzIG9mIHdhc3Rld2F0ZXIgaW5jcmVhc2VkIHRoZSBhYnVuZGFuY2Ugb2YgLi4uIHBoeXRvcGxhbmt0b24gd2VyZSBsaW1pdGVkLiBTYW5kLWZpbHRlcmVkIHNld2FnZSBjb250YWluZWQgbGVzcyBQLCBvcmdhbmljIG1hdHRlciwgYW5kIGFtbW9uaXVtIGJ1dCBtb3JlIG5pdHJhdGUgYW5kIC4uLiBkaWZmZXJpbmcgdHlwZXMgb2Ygc2V3YWdlIGRpc2NoYXJnZSBhbmQgZmVydGlsaXplcnMgY2FuIHByb21vdGUgQ0hBQiBpbnRlbnNpdHkgYW5kIHRveGljaXR5LCB3aGlsZSBjb25jdXJyZW50bHkgLi4uIHNld2FnZSB0aHJvdWdoIHNhbmRzIG11dGVkIGJsb29tIGludGVuc2l0eSBzdWdnZXN0aW5nIHNhbmQtYmVkcyBtYXkgcmVwcmVzZW50IGEgdG9vbCB0byByZW1vdmUga2V5IG51dHJpZW50cyBhbmQgLi4uICggV2F0c29uIGV0IGFsLiwgMjAxNiA7IEJ1bGxlcmphaG4gZXQgYWwuLCAyMDE2IDsgSHVpc21hbiBldCBhbC4sIDIwMTggKSBhbmQgZXhjZXNzaXZlIG51dHJpZW50IGxvYWRpbmcgaXMgdGhlIHByb2Nlc3MgbW9zdCBjb21tb25seSBjaXRlZCBhcyB0aGUgZW52aXJvbm1lbnRhbCBmYWN0b3IgcHJvbW90aW5nIENIQUJzICgifV0%3D. Collectively, HABs drain INR 50,000-100,000 crore yearly from India's lake economies, underscoring the need for resilient strategies (Paerl et al., 2019)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhhbnMgVy4iLCJmYW1pbHkiOiJQYWVybCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLYXJsIEUuIiwiZmFtaWx5IjoiSGF2ZW5zIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hdGhhbiBTLiIsImZhbWlseSI6IkhhbGwifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiVGltb3RoeSBHLiIsImZhbWlseSI6Ik90dGVuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1lbmd5dWFuIiwiZmFtaWx5IjoiWmh1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhhaSIsImZhbWlseSI6Ilh1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikd1YW5nd2VpIiwiZmFtaWx5IjoiWmh1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkJvcWlhbmciLCJmYW1pbHkiOiJRaW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIxMzIzLTE2NTAiLCIxNDQ4LTYwNTkiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwNzEvbWYxODM5MiIsIm1hZyI6IjI5NDIzMjE1NDIiLCJvcGVuYWxleCI6IlcyOTQyMzIxNTQyIn0sInRpdGxlIjoiTWl0aWdhdGluZyBhIGdsb2JhbCBleHBhbnNpb24gb2YgdG94aWMgY3lhbm9iYWN0ZXJpYWwgYmxvb21zOiBjb25mb3VuZGluZyBlZmZlY3RzIGFuZCBjaGFsbGVuZ2VzIHBvc2VkIGJ5IGNsaW1hdGUgY2hhbmdlIiwiYWJzdHJhY3QiOiJNYW5hZ2luZyBhbmQgbWl0aWdhdGluZyB0aGUgZ2xvYmFsIGV4cGFuc2lvbiBvZiB0b3hpYyBjeWFub2JhY3RlcmlhbCBoYXJtZnVsIGFsZ2FsIGJsb29tcyAoQ3lhbm9IQUJzKSBpcyBhIG1ham9yIGNoYWxsZW5nZSBmYWNpbmcgcmVzZWFyY2hlcnMgYW5kIHdhdGVyIHJlc291cmNlIG1hbmFnZXJzLiBWYXJpb3VzIGFwcHJvYWNoZXMsIGluY2x1ZGluZyBudXRyaWVudCBsb2FkIHJlZHVjdGlvbiwgYXJ0aWZpY2lhbCBtaXhpbmcgYW5kIGZsdXNoaW5nLCBvbW5pdm9yb3VzIGZpc2ggcmVtb3ZhbCwgYWxnYWVjaWRlIGFwcGxpY2F0aW9ucyBhbmQgc2VkaW1lbnQgZHJlZGdpbmcsIGhhdmUgYmVlbiB1c2VkIHRvIHJlZHVjZSBibG9vbSBvY2N1cnJlbmNlcy4gSG93ZXZlciwgbWFuYWdlcnMgbm93IGZhY2UgdGhlIGFkZGl0aW9uYWwgY2hhbGxlbmdlIG9mIGhhdmluZyB0byBhZGRyZXNzIHRoZSBlZmZlY3RzIG9mIGNsaW1hdGUgY2hhbmdlIG9uIHdhdGVyc2hlZCBoeWRyb2xvZ2ljYWwgYW5kIG51dHJpZW50IGxvYWQgZHluYW1pY3MsIHdhdGVyIHRlbXBlcmF0dXJlLCBtaXhpbmcgcmVnaW1lIGFuZCBpbnRlcm5hbCBudXRyaWVudCBjeWNsaW5nLiBSaXNpbmcgdGVtcGVyYXR1cmVzIGFuZCBpbmNyZWFzaW5nIGZyZXF1ZW5jaWVzIGFuZCBtYWduaXR1ZGVzIG9mIGV4dHJlbWUgd2VhdGhlciBldmVudHMsIGluY2x1ZGluZyB0cm9waWNhbCBjeWNsb25lcywgZXh0cmF0cm9waWNhbCBzdG9ybXMsIGZsb29kcyBhbmQgZHJvdWdodHMsIGFsbCBwcm9tb3RlIEN5YW5vSEFCcyBhbmQgYWZmZWN0IHRoZSBlZmZpY2FjeSBvZiBlY29zeXN0ZW0gcmVtZWRpYXRpb24gbWVhc3VyZXMuIFRoZXNlIGNsaW1hdGljIGNoYW5nZXMgd2lsbCBsaWtlbHkgcmVxdWlyZSBzZXR0aW5nIHN0cmljdGVyIG51dHJpZW50IChpbmNsdWRpbmcgYm90aCBuaXRyb2dlbiBhbmQgcGhvc3Bob3J1cykgcmVkdWN0aW9uIHRhcmdldHMgZm9yIGJsb29tIGNvbnRyb2wgaW4gYWZmZWN0ZWQgd2F0ZXJzLiBJbiBhZGRpdGlvbiwgdGhlIGVmZmljYWN5IG9mIGN1cnJlbnRseSB1c2VkIG1ldGhvZHMgdG8gcmVkdWNlIEN5YW5vSEFCcyB3aWxsIG5lZWQgdG8gYmUgcmUtZXZhbHVhdGVkIGluIGxpZ2h0IG9mIHRoZSBzeW5lcmdpc3RpYyBlZmZlY3RzIG9mIGNsaW1hdGUgY2hhbmdlIHdpdGggbnV0cmllbnQgZW5yaWNobWVudC4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTA3MS9tZjE4MzkyIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2Nkci5saWIudW5jLmVkdS9kb3dubG9hZHMvMDU3NDIyODM0IiwiaXNzdWVkRGF0ZSI6IjIwMTktMDMtMjUiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTEwIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJNYXJpbmUgYW5kIEZyZXNod2F0ZXIgUmVzZWFyY2giLCJpc3N1ZSI6NSwidm9sdW1lIjo3MSwiaW1wYWN0RmFjdG9yIjoiMi43MCJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjU3OSwibGFzdFBhZ2UiOjU3OX19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJDU0lSTyBQdWJsaXNoaW5nIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjoxMDQsImNpdGVkQnlDb3VudCI6OTF9fSwicXVvdGUiOiJUaGF0IHJlbGF0aW9uc2hpcCBoYXMgc2VydmVkIGFzIHRoZSBmb3VuZGF0aW9uIGZvciBzZXR0aW5nICBudXRyaWVudCBsb2FkaW5nIHRhcmdldHMgaW4gbGFrZXMgYXJvdW5kIHRoZSB3b3JsZCwgYW5kIGNhcmVmdWxseSAgZGVzaWduZWQgZXhwZXJpbWVudHMgbWF5IGVsdWNpZGF0ZSB0byB3aGF0IGV4dGVudCB0ZW1wZXJhdHVyZSAgaW5jcmVhc2UgYWZmZWN0cyB0aGUgdGhyZXNob2xkIGF0IHdoaWNoIHRoZSBmcmVxdWVuY3kgb2Ygb2NjdXIgIHJlbmNlIG9mIEN5YW5vSEFCcyBxdWlja2x5IHJpc2VzIHdpdGggaW5jcmVhc2luZyBOIGFuZCBQLiBDb25jbHVzaW9ucyBSZWNlbnQgc3R1ZGllcyBpbmRpY2F0ZSB0aGF0IHRoZSBzeW5lcmdpc3RpYyBjb21iaW5hdGlvbiBvZiBhbnRocm9wb2dlbmljIG51dHJpZW50IGxvYWRpbmcsIHJpc2luZyB0ZW1wZXJhdHVyZXMsIGVuaGFuY2VkICB2ZXJ0aWNhbCBzdHJhdGlmaWNhdGlvbiwgaW5jcmVhc2VkIHJlc2lkZW5jZSB0aW1lIGFuZCBtb3JlICBleHRyZW1lIGNsaW1hdGljIGNvbmRpdGlvbnMgb3ZlcmFsbCB3aWxsIGZhdm91ciBjeWFub2JhY3RlcmlhbCAgZG9taW5hbmNlIGluIGEgd2lkZSByYW5nZSBvZiBhcXVhdGljIGVjb3N5c3RlbXMuIFRoZSBleHBhbiAgc2lvbiBvZiBDeWFub0hBQnMgaGFzIHNlcmlvdXMgY29uc2VxdWVuY2VzIGZvciBodW1hbiB3YXRlciAgc3VwcGxpZXMsIGZpc2hlcmllcyBhbmQgcmVjcmVhdGlvbmFsIHJlc291cmNlcy4gRWNvc3lzdGVtIHNjYWxlICBwaHlzaWNhbCBjaGVtaWNhbCBiaW90aWMgbWl0aWdhdGlvbiBzdHJhdGVnaWVzIHdpbGwgaGF2ZSB0byAgaW5jb3Jwb3JhdGUgbnV0cmllbnQgKG1vc3Qgb2Z0ZW4gYm90aCBOIGFuZCBQKSBpbnB1dCByZWR1Y3Rpb25zLiAgVGhlIG1hZ25pdHVkZXMgb2YgdGhlc2UgcmVkdWN0aW9ucyB3aWxsIG5lZWQgdG8gYmUgc3lzdGVtICBzcGVjaWZpYyBhbmQgd2lsbCBsaWtlbHkgbmVlZCB0byBiZSBhZGp1c3RlZCB3aXRoIGNoYW5naW5nIGNsaSAgbWF0aWMgY29uZGl0aW9ucy4gU3BlY2lmaWNhbGx5LCB3YXRlciBxdWFsaXR5IG1hbmFnZXJzIHdpbGwgaGF2ZSAgdG8gYWNjb21tb2RhdGUgd2FybWluZyBhbmQgZ3JlYXRlciBoeWRyb2xvZ2ljYWwgdmFyaWFiaWxpdHkgIGFuZCBleHRyZW1lbmVzcyByZXN1bHRpbmcgZnJvbSBjbGltYXRlIGNoYW5nZSBpbiBmb3JtdWxhdGluZyAgdGhlaXIgc3RyYXRlZ2llcy4gQSBrZXkgbG9uZyB0ZXJtIGNvbnRyb2wgd2UgY2FuIGV4ZXJ0IHRvIHJlZHVjZSAgdGhlIHJhdGUgYW5kIGV4dGVudCBvZiBnbG9iYWwgd2FybWluZyBpcyBjdXJiaW5nIGdyZWVuaG91c2UgZ2FzICBlbWlzc2lvbnMuIFdpdGhvdXQgdGhpcyBlc3NlbnRpYWwgc3RlcCwgZnV0dXJlIHdhcm1pbmcgdHJlbmRzLCAgaHlkcm9sb2dpY2FsIGV4dHJlbWVzIChyZWNvcmQgcmFpbmZhbGxzIGFuZCBkcm91Z2h0cykgYW5kIHRoZWlyICBlZmZlY3RzIG9uIGFxdWF0aWMgZWNvc3lzdGVtcyB3aWxsIGNvbnRpbnVlIHRvIHByb21vdGUgb3Bwb3IgIHR1bmlzdGljIEN5YW5vSEFCcy4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6MTQuMTQ5MjMzODE4MDU0MiwibGVmdCI6MTMuMTI4NTk1MzUyMTcyODUyLCJ3aWR0aCI6MzcuMDgyMTcyMzkzNzk4ODMsImhlaWdodCI6MS41MTY2MDIyNzc3NTU3MzczfX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjE1LjQ1MTUwODUyMjAzMzY5MSwibGVmdCI6MTMuMTI4NTQ0ODA3NDM0MDgyLCJ3aWR0aCI6MzcuMDg0ODMxMjM3NzkyOTcsImhlaWdodCI6MS41MTY2MDMyMzE0MzAwNTM3fX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjE2Ljc1Mzc4NDE3OTY4NzUsImxlZnQiOjEzLjEyODU0MjkwMDA4NTQ1LCJ3aWR0aCI6MzcuMDgzMjA5OTkxNDU1MDgsImhlaWdodCI6MS41MTY2MDQxODUxMDQzNzAxfX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjE4LjA1NjA1MzE2MTYyMTA5NCwibGVmdCI6MTMuMTI4NTQ0ODA3NDM0MDgyLCJ3aWR0aCI6MzYuNjE1MDc0MTU3NzE0ODQ0LCJoZWlnaHQiOjEuNTE2NjA0MTg1MTA0MzcwMX19LHsicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjoxOS40NjgzNzgwNjcwMTY2LCJsZWZ0IjoxMy4xMjg1NDQ4MDc0MzQwODIsIndpZHRoIjozNC4yOTA1MzExNTg0NDcyNjYsImhlaWdodCI6MS4yNDUwMTEyMTA0NDE1ODk0fX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjIyLjIyNjE0Mjg4MzMwMDc4LCJsZWZ0IjoxMy4xMjg1NTA1Mjk0Nzk5OCwid2lkdGgiOjcuNjQ5ODczNzMzNTIwNTA4LCJoZWlnaHQiOjEuMTM1NDgzMTQ1NzEzODA2Mn19LHsicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjoyMy43MzQ4NDk5Mjk4MDk1NywibGVmdCI6MTMuMTI4NTM1MjcwNjkwOTE4LCJ3aWR0aCI6MzcuMDg5Mjg2ODA0MTk5MjIsImhlaWdodCI6MjYuMDg4NjQwMjEzMDEyNjk1fX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNTg5In19XQ%3D%3D. Integrated interventions riparian buffers, early warning systems, and diversified livelihoods offer pathways to mitigate these tolls, fostering equitable recovery. Globally, economic evaluations of HABs estimate annual losses of approximately $100 million in the United States, with 37% attributed to commercial fishery closures and 13% to lost recreation and tourism opportunities (Berdalet et al., 2017)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkVsaXNhIiwiZmFtaWx5IjoiQmVyZGFsZXQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUmFwaGFlbCBNLiIsImZhbWlseSI6Ikt1ZGVsYSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJFZCIsImZhbWlseSI6IlVyYmFuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhlbnJpayIsImZhbWlseSI6IkVuZXZvbGRzZW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTmVpbCBTLiIsImZhbWlseSI6IkJhbmFzIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkVpbGVlbiIsImZhbWlseSI6IkJyZXNuYW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTWljaGVsZSBBLiIsImZhbWlseSI6IkJ1cmZvcmQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiS2VpdGgiLCJmYW1pbHkiOiJEYXZpZHNvbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJDaHJpc3RvcGhlciBKLiIsImZhbWlseSI6IkdvYmxlciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJCZW5ndCIsImZhbWlseSI6Ikthcmxzb24ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUG8gVGVlbiIsImZhbWlseSI6IkxpbSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJMaW5jb2xuIiwiZmFtaWx5IjoiTWFjS2VuemllIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1hcmluYSIsImZhbWlseSI6Ik1vbnRyZXNvciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJWZXJhIEwuIiwiZmFtaWx5IjoiVHJhaW5lciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJHaXJlcyIsImZhbWlseSI6IlVzdXAifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiS2Vkb25nIiwiZmFtaWx5IjoiWWluIn19XSwiaWRzIjp7Imlzc24iOlsiMTA0Mi04Mjc1IiwiMjM3Ny02MTdYIl0sImlzYm4iOltdLCJkb2kiOiIxMC41NjcwL29jZWFub2cuMjAxNy4xMTEiLCJtYWciOiIyNTk3NjY0NzI3Iiwib3BlbmFsZXgiOiJXMjU5NzY2NDcyNyIsIm9wZW5BY2Nlc3NJZCI6IjE2Njk1MTg2NjgzNTA1MjM4MTQ1In0sInRpdGxlIjoiR2xvYmFsSEFCOiBBIE5ldyBQcm9ncmFtIHRvIFByb21vdGUgSW50ZXJuYXRpb25hbCBSZXNlYXJjaCwgT2JzZXJ2YXRpb25zLCBhbmQgTW9kZWxpbmcgb2YgSGFybWZ1bCBBbGdhbCBCbG9vbXMgaW4gQXF1YXRpYyBTeXN0ZW1zIiwiYWJzdHJhY3QiOiJCZXJkYWxldCwgRWxpc2EgLi4uIGV0IGFsLi0tIFNwZWNpYWwgaXNzdWUgb24gSW50ZXJuYXRpb25hbCBDb29wZXJhdGlvbiBpbiBIYXJtZnVsIEFsZ2FsIEJsb29tIFNjaWVuY2UuLS0gMTIgcGFnZXMsIDYgZmlndXJlcyIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC41NjcwL29jZWFub2cuMjAxNy4xMTEiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vdG9zLm9yZy9vY2Vhbm9ncmFwaHkvYXNzZXRzL2RvY3MvMzAtMV9iZXJkYWxldDMucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMTctMDMtMDEiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTEwIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJPY2Vhbm9ncmFwaHkiLCJpc3N1ZSI6MSwidm9sdW1lIjozMCwiaW1wYWN0RmFjdG9yIjoiMi45MyJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjcwLCJsYXN0UGFnZSI6ODF9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiT2NlYW5vZ3JhcGh5IFNvY2lldHkifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjI2LCJjaXRlZEJ5Q291bnQiOjMyfX0sInF1b3RlIjoiVGhlICBlY29ub21pYyBlZmZlY3RzIG9mIEhBQnMgYXJpc2UgZnJvbSBwdWItbGljIGhlYWx0aCBjb3N0cywgY29tbWVyY2lhbCBmaXNoZXJ5IGFuZCAgYXF1YWN1bHR1cmUgb3BlcmF0aW9uIGNsb3N1cmVzIGFuZCBmaXNoICBraWxscywgcG9zc2libGUgbWVkaXVtLSBhbmQgbG9uZy10ZXJtICBkZWNsaW5lcyBpbiBjb2FzdGFsIGFuZCBtYXJpbmUgcmVjcmVhdGlvbiAgYW5kIHRvdXJpc20sIGFuZCB0aGUgY29zdHMgb2YgaW5zdXJhbmNlLCAgbW9uaXRvcmluZywgbWFuYWdlbWVudCwgYW5kIG1pdGlnYXRpb24gIChNb3JnYW4gZXQgYWwuLCAyMDEwKS4gQWdncmVnYXRpbmcgZWNvLW5vbWljIGVmZmVjdHPigJRib3RoIHdpdGhpbiBhbmQgYWNyb3NzICB0aGVzZSBjYXRlZ29yaWVz4oCUY2FuIGFsc28gYmUgcHJvYmxlbS1hdGljIHdpdGggYXZhaWxhYmxlIGRhdGEgYW5kIGN1cnJlbnRseSB1c2VkICBtZXRob2RvbG9naWVzIChIb2FnbGFuZCBldCBhbC4sIDIwMDIpLiAgQW4gZXhhbXBsZSBvZiBhbiBlY29ub21pYyBldmFsdWF0aW9uICBvZiBIQUJzIGF0IGEgbmF0aW9uYWwgbGV2ZWwgaXMgZ2l2ZW4gYnkgIEFuZGVyc29uIGV0IGFsLiAoMjAwMCksIHdobyBlc3RpbWF0ZWQgdGhlICBlY29ub21pYyBlZmZlY3RzIG9mIEhBQnMgaW4gdGhlIFVuaXRlZCAgU3RhdGVzIHRvIGJlICQxMDAgbWlsbGlvbiBwZXIgeWVhciAoYXMgbGF0ZXIgIHRyYW5zbGF0ZWQgaW50byAyMDEyIGRvbGxhcnMpLCBicm9rZW4gZG93biAgYXMgZm9sbG93czogNDUlIGZvciBwdWJsaWMgaGVhbHRoIGNvc3RzLCAgMzclIGZvciB0aGUgY29zdHMgb2YgY2xvc3VyZXMgYW5kIGxvc3NlcyAgZXhwZXJpZW5jZWQgYnkgY29tbWVyY2lhbCBmaXNoZXJpZXMsICAxMyUgZnJvbSB0aGUgaW1wYWN0IG9uIGxvc3QgcmVjcmVhdGlvbiAgYW5kIHRvdXJpc20gb3Bwb3J0dW5pdGllcywgYW5kIDQlIGZyb20gRklHVVJFIDYuIE1hc3NpdmUgZmlzaCBtb3J0YWxpdGllcyBpbiBKYWthcnRhLCBJbmRvbmVzaWEuIFBob3RvIGNyZWRpdHM6IFR1bXBhayBTaWRhYnV0YXIgKEhBTjUzKSBtb25pdG9yaW5nIGFuZCBtYW5hZ2VtZW50IGNvc3RzLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjExLjgxOTQ0NDY1NjM3MjA3LCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzNDk0NTI5NzI0MTIxLCJoZWlnaHQiOjEuNzAzMjgyNTk0NjgwNzg2MX19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjEzLjUyMzk4OTY3NzQyOTIsImxlZnQiOjY3LjI1NDg5ODA3MTI4OTA2LCJ3aWR0aCI6MjYuODY0Mzc0MTYwNzY2NiwiaGVpZ2h0IjoxLjcwMzI4MjU5NDY4MDc4NjF9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjoxNS4yMjg1MzU2NTIxNjA2NDUsImxlZnQiOjY3LjI1NDg5ODA3MTI4OTA2LCJ3aWR0aCI6MjcuMjMyMDAyMjU4MzAwNzgsImhlaWdodCI6MS43MDMyODM1NDgzNTUxMDI1fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6MTYuOTMzMDgwNjczMjE3NzczLCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzMzYzNjg1NjA3OTEsImhlaWdodCI6MS43MDMyODM1NDgzNTUxMDI1fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6MTguNjM3NjI2NjQ3OTQ5MjIsImxlZnQiOjY3LjI1NDg5ODA3MTI4OTA2LCJ3aWR0aCI6MjcuMjM1Mjk0MzQyMDQxMDE2LCJoZWlnaHQiOjEuNzAzMjgzNTQ4MzU1MTAyNX19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjIwLjM0MjE3MDcxNTMzMjAzLCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzMzk4NTkwMDg3ODkwNiwiaGVpZ2h0IjoxLjcwMzI4MzU0ODM1NTEwMjV9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjoyMi4wNDY3MTY2OTAwNjM0NzcsImxlZnQiOjY3LjI1NDg5ODA3MTI4OTA2LCJ3aWR0aCI6MjcuMjMyMDExNzk1MDQzOTQ1LCJoZWlnaHQiOjEuNzAzMjgzNTQ4MzU1MTAyNX19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjIzLjc1MTI2MjY2NDc5NDkyMiwibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzIwNDIzMTI2MjIwNywiaGVpZ2h0IjoxLjcwMzI4MzU0ODM1NTEwMjV9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjoyNS40NTU4MDg2Mzk1MjYzNjcsImxlZnQiOjY3LjI1NDg5ODA3MTI4OTA2LCJ3aWR0aCI6MjYuODYzMDY3NjI2OTUzMTI1LCJoZWlnaHQiOjEuNzAzMjgzNTQ4MzU1MTAyNX19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjI3LjE2MDM1MjcwNjkwOTE4LCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzNTQzMzU3ODQ5MTIxLCJoZWlnaHQiOjEuNzAzMjgzNTQ4MzU1MTAyNX19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjI4Ljg2NDg5ODY4MTY0MDYyNSwibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNi44NjQzOTUxNDE2MDE1NjIsImhlaWdodCI6MS43MDMyODM1NDgzNTUxMDI1fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6MzAuNTY5NDQyNzQ5MDIzNDM4LCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzNDkyNDMxNjQwNjI1LCJoZWlnaHQiOjEuNzAzMjgzNTQ4MzU1MTAyNX19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjMyLjI3Mzk5MDYzMTEwMzUxNiwibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzQ3NzU1NDMyMTI4OSwiaGVpZ2h0IjoxLjcwMzI4MzU0ODM1NTEwMjV9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjozMy45Nzg1MzQ2OTg0ODYzMywibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzQzMzY4NTMwMjczNDQsImhlaWdodCI6MS43MDMyODU0NTU3MDM3MzU0fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6MzUuNjgzMDc4NzY1ODY5MTQsImxlZnQiOjY3LjI1NDg5ODA3MTI4OTA2LCJ3aWR0aCI6MjcuMjMyMTYwNTY4MjM3MzA1LCJoZWlnaHQiOjEuNzAzMjg1NDU1NzAzNzM1NH19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjM3LjM4NzYyMjgzMzI1MTk1LCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzMzY2NTQ2NjMwODU5NCwiaGVpZ2h0IjoxLjcwMzI4NTQ1NTcwMzczNTR9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjozOS4wOTIxNzA3MTUzMzIwMywibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzY1Nzk4OTUwMTk1MywiaGVpZ2h0IjoxLjcwMzI4NTQ1NTcwMzczNTR9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo0MC43OTY3MTQ3ODI3MTQ4NDQsImxlZnQiOjY3LjI1NDg5ODA3MTI4OTA2LCJ3aWR0aCI6MjcuMjM2NzE3MjI0MTIxMDk0LCJoZWlnaHQiOjEuNzAzMjg1NDU1NzAzNzM1NH19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjQyLjUwMTI2MjY2NDc5NDkyLCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzMzYyNzMxOTMzNTkzOCwiaGVpZ2h0IjoxLjcwMzI4NTQ1NTcwMzczNTR9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo0NC4yMDU4MDY3MzIxNzc3MzQsImxlZnQiOjY3LjI1NDg5ODA3MTI4OTA2LCJ3aWR0aCI6MjcuMjMwNTQ1MDQzOTQ1MzEyLCJoZWlnaHQiOjEuNzAzMjg1NDU1NzAzNzM1NH19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjQ1LjkxMDM1MDc5OTU2MDU1LCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzODU0NDQ2NDExMTMyOCwiaGVpZ2h0IjoxLjcwMzI4NTQ1NTcwMzczNTR9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo0Ny42MTQ4OTQ4NjY5NDMzNiwibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzQxNDYxMTgxNjQwNjIsImhlaWdodCI6MS43MDMyODU0NTU3MDM3MzU0fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6NDkuMzE5NDQyNzQ5MDIzNDQsImxlZnQiOjY3LjI1NDg5ODA3MTI4OTA2LCJ3aWR0aCI6MjcuMjM1OTQwOTMzMjI3NTQsImhlaWdodCI6MS43MDMyODU0NTU3MDM3MzU0fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6NTEuMDIzOTkwNjMxMTAzNTE2LCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzNjkwNzk1ODk4NDM3NSwiaGVpZ2h0IjoxLjcwMzI4NTQ1NTcwMzczNTR9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo5Mi4yNzg3NzA0NDY3NzczNCwibGVmdCI6OC44MjM1MjkyNDM0NjkyMzgsIndpZHRoIjo1Ni4yNjM5MTYwMTU2MjUsImhlaWdodCI6MS4yMDkwODk1MTc1OTMzODM4fX0seyJwYWdlTnVtYmVyIjo5LCJiYm94Ijp7InRvcCI6NS4wMDEyNjI2NjQ3OTQ5MjIsImxlZnQiOjUuODgyMzUyODI4OTc5NDkyLCJ3aWR0aCI6MjcuMjM1Mjk0MzQyMDQxMDE2LCJoZWlnaHQiOjEuNzAzMjgyNTk0NjgwNzg2MX19XX0sImNodW5rSWQiOiI5NjIxNjM5Zi0zYWY5LTRkOGMtYmIwNy0wMzUwMmViMjEwN2YiLCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI3OCJ9fV0%3D. Beyond these direct expenditures, the depreciation of real estate values constitutes a substantial, often overlooked economic burden, as lakefront properties experience significant devaluation when water quality deteriorates (Sampat et al., 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFwb29ydmEgTS4iLCJmYW1pbHkiOiJTYW1wYXQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW5kcmVhIiwiZmFtaWx5IjoiSGlja3MifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiR2VyYXJkbyBKLiIsImZhbWlseSI6IlJ1aXrigJBNZXJjYWRvIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlbDrcKtY3RvciBNLiIsImZhbWlseSI6IlphdmFsYSJ9fV0sImlkcyI6eyJpc3NuIjpbIjA5MjEtMzQ0OSIsIjE4NzktMDY1OCJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMTAxNi9qLnJlc2NvbnJlYy4yMDIwLjEwNTE5OSIsIm1hZyI6IjMwOTQ2MjUxMDQiLCJwbWlkIjoiMzM3NDYzNjEiLCJvcGVuYWxleCI6IlczMDk0NjI1MTA0In0sInRpdGxlIjoiVmFsdWluZyBlY29ub21pYyBpbXBhY3QgcmVkdWN0aW9ucyBvZiBudXRyaWVudCBwb2xsdXRpb24gZnJvbSBsaXZlc3RvY2sgd2FzdGUiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAxNi9qLnJlc2NvbnJlYy4yMDIwLjEwNTE5OSIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cubmNiaS5ubG0ubmloLmdvdi9wbWMvYXJ0aWNsZXMvNzk3MDUwNSIsImlzc3VlZERhdGUiOiIyMDIwLTEwLTE1IiwiYWNjZXNzZWREYXRlIjoiMjAyNi0wMSIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiUmVzb3VyY2VzIENvbnNlcnZhdGlvbiBhbmQgUmVjeWNsaW5nIiwidm9sdW1lIjoxNjQsImltcGFjdEZhY3RvciI6IjExLjAxIn0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTA1MTk5LCJsYXN0UGFnZSI6MTA1MTk5fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkVsc2V2aWVyIEJWIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjozOSwiY2l0ZWRCeUNvdW50Ijo1N319LCJxdW90ZSI6IlNlY3Rpb24gVGl0bGU6IFZhbHVpbmcgZWNvbm9taWMgaW1wYWN0IHJlZHVjdGlvbnMgb2YgbnV0cmllbnQgcG9sbHV0aW9uIGZyb20gbGl2ZXN0b2NrIHdhc3RlID4gMS4gSW50cm9kdWN0aW9uXG5Db250ZW50OlxuSW4gdGhlIFN0YXRlIG9mIEZsb3JpZGEsIEhBQnMgaGF2ZSByZXN1bHRlZCBpbiBhIG1vbnRobHkgbG9zcyBvZiAyLjggYW5kIDMuNyBtaWxsaW9uIFVTRCBjb3JyZXNwb25kaW5nIHRvIHJlc3RhdXJhbnQgYW5kIGxvZGdpbmcgcmV2ZW51ZSwgcmVzcGVjdGl2ZWx5ICggW0xhcmtpbiBhbmQgQWRhbXMsIDIwMDddKCkgKS4gRnJlcXVlbnQgb2NjdXJyZW5jZSBvZiBIQUJzIGFsc28gbG93ZXJzIHByb3BlcnR5IHZhbHVlcyBvZiBsYWtlZnJvbnQgcHJvcGVydGllcy4gVGhlIGxvc3MgaW4gcHJvcGVydHkgdmFsdWVzIGFyZSB0aGUgbGFyZ2VzdCBpbXBhY3QgYmVhcmVycyBvZiBldXRyb3BoaWNhdGlvbiB3aXRoIGFuIGVzdGltYXRlZCBhdmVyYWdlIGVjb25vbWljIGxvc3Mgb2YgMS42IGJpbGxpb24gVVNEIGFubnVhbGx5ICggW0RvZGRzIGV0IGFsLiwgMjAwOV0oKSApLiIsImV4dGVybmFsQ2FjaGVJZCI6ImUyMDgyOWRiLTYwZWQtM2YwOC1iZWNlLTcyY2JkYTI4YTIwOCJ9XQ%3D%3D. These balance sheet effects extend to commercial enterprises, where ongoing losses tied to asset depreciation ultimately lead to reductions in broader real estate values (Devlin et al., 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjEsInN1YnR5cGUiOnt9LCJjb250cmlidXRvcnMiOlt7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBZGFtIFQuIiwiZmFtaWx5IjoiRGV2bGluIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkppYXlpIiwiZmFtaWx5IjoiUGFuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik0uIE1hbmp1ciIsImZhbWlseSI6IlNoYWgifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQWxpeXUiLCJmYW1pbHkiOiJOdWh1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Iktlbm5ldGggVy4iLCJmYW1pbHkiOiJDdW1taW5zIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkNsYXVkaW8gVmFsZG92aW5vcyIsImZhbWlseSI6IlphcmdlcyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQYWJsbyIsImZhbWlseSI6IkZpZXJybyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJWaXZpYW5hIiwiZmFtaWx5IjoiT2xtb3MifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQWJkdWwtUmFobWFuIiwiZmFtaWx5IjoiRGlyaXN1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkpvaG4gT3ZpZSIsImZhbWlseSI6Ik9sb211a29ybyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJOYXRpdmlkYWRlIiwiZmFtaWx5IjoiVmllaXJhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFuYSIsImZhbWlseSI6IlNvYXJlcyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJDYXJsb3MiLCJmYW1pbHkiOiJQaW5oZWlybyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNYXR0aGV3IEYuIiwiZmFtaWx5IjoiQmluZ2hhbSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJKYXNvbiIsImZhbWlseSI6Iktpbm5lbGwifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiV3UiLCJmYW1pbHkiOiJCbyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJKaW5tdSIsImZhbWlseSI6IlpoYW5nIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllpbmRpIiwiZmFtaWx5IjoiWmhhbyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJIdW1iZXJ0byBBbHZlcyIsImZhbWlseSI6IkJhcmJvc2EifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiRnJhbmtsaW4iLCJmYW1pbHkiOiJQYXJlZGVz4oCQVHJlam8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiVCBMYWtzaG1pIiwiZmFtaWx5IjoiS3VtYXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiS2lyYW4iLCJmYW1pbHkiOiJUaGFrdXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQ2F0YXJpbmEiLCJmYW1pbHkiOiJCdXJpdGkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2NvdHQgQS4iLCJmYW1pbHkiOiJXZWxscyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJPc2NhciBPbW9uZGkiLCJmYW1pbHkiOiJEb25kZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJDYXJlbiBOYXZhbGlhIiwiZmFtaWx5IjoiQXRhbGl0c2EifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiRC4iLCJmYW1pbHkiOiJTYW5pIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNhbXBzb24iLCJmYW1pbHkiOiJBaG1lZCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBbWFuaW5nIiwiZmFtaWx5IjoiS3dha3UifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiR2V2b3JnIiwiZmFtaWx5IjoiQWRqZWkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2F0YXJ1cGEiLCJmYW1pbHkiOiJTaW1vbnlhbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTbmlnZGhhIiwiZmFtaWx5IjoiR2hvc2gifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUHJhc2FubmEiLCJmYW1pbHkiOiJDaGF0dGVyamVlIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNoaXZhIiwiZmFtaWx5IjoiR2hvcmEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQmVsZGEiLCJmYW1pbHkiOiJQcmFzYWQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiS2V0aGxhdGxvZ2lsZSIsImZhbWlseSI6Ik1vc2VwZWxlIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ill1c3VmIiwiZmFtaWx5IjoiU3VsZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJVaXLDoSIsImZhbWlseSI6Ik9saXZlaXJhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1hcmlhIEpvw6NvIiwiZmFtaWx5IjoiVmllaXJhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkZsZXVyIiwiZmFtaWx5IjoiRGUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTWluIiwiZmFtaWx5IjoiTWF4In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikdldm9yZyIsImZhbWlseSI6IlNpbW9ueWFuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNhdGFydXBhIiwiZmFtaWx5IjoiR2hvc2gifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU25pZ2RoYSIsImZhbWlseSI6IkNoYXR0ZXJqZWUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUHJhc2FubmEiLCJmYW1pbHkiOiJQcmFzYWQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2F0YXJ1cGEiLCJmYW1pbHkiOiJHaG9zaCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJHaG9yYSIsImZhbWlseSI6IlByYXNhZCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQcmFzYW5uYSIsImZhbWlseSI6IlBhbCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLd2FrdSIsImZhbWlseSI6IkFnb2R6byJ9fV0sImlkcyI6eyJpc3NuIjpbXSwiaXNibiI6W10sImRvaSI6IjEwLjU3NzIvaW50ZWNob3Blbi44NzQ2MyIsIm1hZyI6IjMxMjg1ODc0NjkiLCJvcGVuYWxleCI6IlczMTI4NTg3NDY5Iiwib3BlbkFjY2Vzc0lkIjoiNTgzNDQyMzk4ODg0OTA5OTEwMyJ9LCJ0aXRsZSI6IklubGFuZCBXYXRlcnMgLSBEeW5hbWljcyBhbmQgRWNvbG9neSIsImFic3RyYWN0IjoiSW5sYW5kIHdhdGVycywgbGFrZXMsIHJpdmVycywgYW5kIHRoZWlyIGNvbm5lY3RlZCB3ZXRsYW5kcyBhcmUgdGhlIG1vc3QgaW1wb3J0YW50IGFuZCB0aGUgbW9zdCB2dWxuZXJhYmxlIHNvdXJjZXMgb2YgZnJlc2h3YXRlciBvbiB0aGUgcGxhbmV0LiBUaGUgZWNvbG9neSBvZiB0aGVzZSBzeXN0ZW1zIGluY2x1ZGVzIGJpb2xvZ3kgYXMgd2VsbCBhcyBodW1hbiBwb3B1bGF0aW9ucyBhbmQgY2l2aWxpemF0aW9uLiBJbmxhbmQgd2F0ZXJzIGFuZCB3ZXRsYW5kcyBhcmUgaGlnaGx5IHN1c2NlcHRpYmxlIHRvIGNoZW1pY2FsIGFuZCBiaW9sb2dpY2FsIHBvbGx1dGFudHMgZnJvbSBuYXR1cmFsIG9yIGh1bWFuIHNvdXJjZXMsIGNoYW5nZXMgaW4gd2F0ZXJzaGVkIGR5bmFtaWNzIGR1ZSB0byB0aGUgZXN0YWJsaXNobWVudCBvZiBkYW1zIGFuZCByZXNlcnZvaXJzLCBhbmQgbGFuZCB1c2UgY2hhbmdlcyBmcm9tIGFncmljdWx0dXJlIGFuZCBpbmR1c3RyeS4gVGhpcyBib29rIHByb3ZpZGVzIGEgY29tcHJlaGVuc2l2ZSByZXZpZXcgb2YgaXNzdWVzIGludm9sdmluZyBpbmxhbmQgd2F0ZXJzIGFuZCBkaXNjdXNzZXMgbWFueSB3b3JsZHdpZGUgaW5sYW5kIHdhdGVyIHN5c3RlbXMuIFRoZSBtYWluIHRvcGljcyBvZiB0aGlzIHRleHQgYXJlIHdhdGVyIHF1YWxpdHkgaW52ZXN0aWdhdGlvbiwgYW5hbHlzZXMgb2YgdGhlIGVjb2xvZ3kgb2YgaW5sYW5kIHdhdGVyIHN5c3RlbXMsIHJlbW90ZSBzZW5zaW5nIG9ic2VydmF0aW9uIGFuZCBudW1lcmljYWwgbW9kZWxpbmcgbWV0aG9kcywgYW5kIGJpb2RpdmVyc2l0eSBpbnZlc3RpZ2F0aW9ucy4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuNTc3Mi9pbnRlY2hvcGVuLjg3NDYzIiwicmVzb3VyY2VVcmwiOiJodHRwczovL210cy5pbnRlY2hvcGVuLmNvbS9zdG9yYWdlL2Jvb2tzLzk2NjAvYXV0aG9yc19ib29rL2F1dGhvcnNfYm9vay5wZGYiLCJpc3N1ZWREYXRlIjoiMjAyMC0wNC0wMSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkludGVjaE9wZW4gZUJvb2tzIiwiaW1wYWN0RmFjdG9yIjoiMS4wMSJ9LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJJbnRlY2hPcGVuIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjozMzgsImNpdGVkQnlDb3VudCI6Mjd9fSwicXVvdGUiOiJBZGRpdGlvbmFsbHksIHNpbmNlIGFmZmVjdGVkIGJ1c2luZXNzZXPigJkgdmFsdWVzICBhcmUgbW9zdCBsaWtlbHkgdGllZCB0byB0aGVpciBhc3NldHMgYW5kIHRoZSByZWFsIGVzdGF0ZSB0aGV5IG9jY3VweSAoZm9yIGV4YW1wbGUsICBhIG1hcmluYSBpcyBub3QgZWFzaWx5IGNvbnZlcnRlZCB0byBzb21lIG90aGVyIHVzZSksIG9uLWdvaW5nIGJhbGFuY2Ugc2hlZXQgZWZmZWN0cyAgd291bGQgdWx0aW1hdGVseSBsZWFkIHRvIHJlZHVjdGlvbnMgaW4gY29tbWVyY2lhbCByZWFsIGVzdGF0ZSB2YWx1ZXMuIFRoZXJlIGFyZSBtYW55IGNoYWxsZW5nZXMgdG8gdW5kZXJzdGFuZGluZyB0aGUgaW1wbGljYXRpb25zIG9mIGNoYW5nZXMgaW4gIHRvdXJpc20gZnJvbSBIQUJzLiBUaGUgY2xlYXJlc3QgY2hhbGxlbmdlIG9ic3RydWN0aW5nIGEgcHJlY2lzZSBhbmFseXNpcyBvZiB0aGVzZSAgaW1wYWN0cyBpcyBhIGxhY2sgb2YgZGF0YSBlaXRoZXIgb24gdGhlIGFtb3VudCBvZiB0b3VyaXNtIGF0IHJpc2sgb3IgdGhlIHNwZWNpZmljICBpbXBhY3Qgb2YgSEFCcyBvbiB0b3VyaXNtLiBGb3IgZXhhbXBsZSwgd2hpbGUgY291bnR5LWxldmVsIGRhdGEgZXhpc3RzIGZvciB0b3RhbCAgZXhwZW5kaXR1cmVzIG9uIHRvdXJpc20sIHRoaXMgaW5jbHVkZXMgdG91cmlzbSB3aGljaCB3b3VsZCBub3QgYmUgaW50ZXJydXB0ZWQgYnkgIEhBQnMgb3Igb3RoZXIgZGlzY291cmFnaW5nIGZhY3RvcnMuIEFuIGFkZGl0aW9uYWwgY2hhbGxlbmdlIHJlbGF0ZXMgdG8gdGhlIGRpc3RpbmN0aW9uIGJldHdlZW4gZWNvbm9taWMgYmVuZWZpdHMgICh3aWxsaW5nbmVzcyB0byBwYXkpIGFuZCBlY29ub21pYyBpbXBhY3RzIChleHBlbmRpdHVyZXMpLCBhbmQgdGhlIG1lYXN1cmUtbWVudCBvZiB0aGUgZWNvbm9taWMgYmVuZWZpdHMgdGhhdCBhcmlzZSBmcm9tIGVjb25vbWljIGltcGFjdHMgKHByb2ZpdHMpLiBGb3IgIGV4YW1wbGUsIGNvbnNpZGVyIGEgcmVzdGF1cmFudCBvd25lciB3aG8gbG9zZXMgJDEwLDAwMCBpbiByZXZlbnVlIGJlY2F1c2Ugb2YgYSAgSEFCLiBUaGUgb3duZXLigJlzIHdpbGxpbmduZXNzIHRvIHBheSB0byByZWNvdmVyIHRoYXQgcmV2ZW51ZTsgaXMgKHJvdWdobHkgc3BlYWtpbmcpICB0aGUgbG9zdCBwcm9maXQgb24gdGhhdCByZXZlbnVlLiBUaGlzIGlzIG1vcmUgZGlmZmljdWx0IHRvIGlkZW50aWZ5IHRoYW4gbG9zdCByZXZlbnVlLiAgIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjg5LCJiYm94Ijp7InRvcCI6NTkuNDQyODY3Mjc5MDUyNzM0LCJsZWZ0IjoxNC43MjI4NTM2NjA1ODM0OTYsIndpZHRoIjo2OC43NDgwMDEwOTg2MzI4MSwiaGVpZ2h0IjoxLjkwNTAwMDgwNTg1NDc5NzR9fSx7InBhZ2VOdW1iZXIiOjg5LCJiYm94Ijp7InRvcCI6NjEuMDcxMDc1NDM5NDUzMTI1LCJsZWZ0IjoxNC43MjI4NTM2NjA1ODM0OTYsIndpZHRoIjo2Ni4zOTYzNDcwNDU4OTg0NCwiaGVpZ2h0IjoxLjkwNTAwMDgwNTg1NDc5NzR9fSx7InBhZ2VOdW1iZXIiOjg5LCJiYm94Ijp7InRvcCI6NjIuNjk5Mjc1OTcwNDU4OTg0LCJsZWZ0IjoxNC43MjI4NTM2NjA1ODM0OTYsIndpZHRoIjo2OS4yNjkzMDk5OTc1NTg2LCJoZWlnaHQiOjEuOTA1MDAwODA1ODU0Nzk3NH19LHsicGFnZU51bWJlciI6ODksImJib3giOnsidG9wIjo2NC4zMjc0ODQxMzA4NTkzOCwibGVmdCI6MTQuNzIyODUzNjYwNTgzNDk2LCJ3aWR0aCI6NTcuNDAwNTA1MDY1OTE3OTcsImhlaWdodCI6MS45MDUwMDA4MDU4NTQ3OTc0fX0seyJwYWdlTnVtYmVyIjo4OSwiYmJveCI6eyJ0b3AiOjY1Ljk1NTY4ODQ3NjU2MjUsImxlZnQiOjE3LjUsIndpZHRoIjo2My4wOTU2OTkzMTAzMDI3MzQsImhlaWdodCI6MS45MDUwMDA4MDU4NTQ3OTc0fX0seyJwYWdlTnVtYmVyIjo4OSwiYmJveCI6eyJ0b3AiOjY3LjU4Mzg5MjgyMjI2NTYyLCJsZWZ0IjoxNC43MjI4NTM2NjA1ODM0OTYsIndpZHRoIjo2OC41MjI1OTgyNjY2MDE1NiwiaGVpZ2h0IjoxLjkwNDk5NjYzMzUyOTY2M319LHsicGFnZU51bWJlciI6ODksImJib3giOnsidG9wIjo2OS4yMTIwODk1Mzg1NzQyMiwibGVmdCI6MTQuNzIyODUzNjYwNTgzNDk2LCJ3aWR0aCI6NjUuNDEyMjc3MjIxNjc5NjksImhlaWdodCI6MS45MDUwMDA4MDU4NTQ3OTc0fX0seyJwYWdlTnVtYmVyIjo4OSwiYmJveCI6eyJ0b3AiOjcwLjg0MDI5Mzg4NDI3NzM0LCJsZWZ0IjoxNC43MjI4NTM2NjA1ODM0OTYsIndpZHRoIjo2Ny43NzAwMTk1MzEyNSwiaGVpZ2h0IjoxLjkwNTAwNTA5NzM4OTIyMTJ9fSx7InBhZ2VOdW1iZXIiOjg5LCJiYm94Ijp7InRvcCI6NzIuNDY4NDk4MjI5OTgwNDcsImxlZnQiOjE0LjcyMjg1MzY2MDU4MzQ5Niwid2lkdGgiOjY5LjM4NjMyOTY1MDg3ODksImhlaWdodCI6MS45MDUwMDUwOTczODkyMjEyfX0seyJwYWdlTnVtYmVyIjo4OSwiYmJveCI6eyJ0b3AiOjc0LjA5NjcwMjU3NTY4MzYsImxlZnQiOjE0LjcyMjg1MzY2MDU4MzQ5Niwid2lkdGgiOjMwLjEzNDg4MDA2NTkxNzk3LCJoZWlnaHQiOjEuOTA1MDA1MDk3Mzg5MjIxMn19LHsicGFnZU51bWJlciI6ODksImJib3giOnsidG9wIjo3NS43MjQ5MTQ1NTA3ODEyNSwibGVmdCI6MTcuNSwid2lkdGgiOjY0LjQxMDc5NzExOTE0MDYyLCJoZWlnaHQiOjEuOTA1MDA1MDk3Mzg5MjIxMn19LHsicGFnZU51bWJlciI6ODksImJib3giOnsidG9wIjo3Ny4zNTMxMTEyNjcwODk4NCwibGVmdCI6MTQuNzIyODUzNjYwNTgzNDk2LCJ3aWR0aCI6NjQuODU1NjkwMDAyNDQxNCwiaGVpZ2h0IjoxLjkwNTAwNTA5NzM4OTIyMTJ9fSx7InBhZ2VOdW1iZXIiOjg5LCJiYm94Ijp7InRvcCI6NzguOTgxMzE1NjEyNzkyOTcsImxlZnQiOjE0LjcyMjg1MzY2MDU4MzQ5Niwid2lkdGgiOjY2LjAxOTg1MTY4NDU3MDMxLCJoZWlnaHQiOjEuOTA1MDA1MDk3Mzg5MjIxMn19LHsicGFnZU51bWJlciI6ODksImJib3giOnsidG9wIjo4MC42MDk1Mjc1ODc4OTA2MiwibGVmdCI6MTQuNzIyODUzNjYwNTgzNDk2LCJ3aWR0aCI6NjcuNzg3NjQzNDMyNjE3MTksImhlaWdodCI6MS45MDUwMDUwOTczODkyMjEyfX0seyJwYWdlTnVtYmVyIjo4OSwiYmJveCI6eyJ0b3AiOjgyLjIzNzcyNDMwNDE5OTIyLCJsZWZ0IjoxNC43MjI4NTM2NjA1ODM0OTYsIndpZHRoIjo2OS43Mzk2OTI2ODc5ODgyOCwiaGVpZ2h0IjoxLjkwNTAwNTA5NzM4OTIyMTJ9fSx7InBhZ2VOdW1iZXIiOjg5LCJiYm94Ijp7InRvcCI6ODMuODY1OTM2Mjc5Mjk2ODgsImxlZnQiOjE0LjcyMjg1MzY2MDU4MzQ5Niwid2lkdGgiOjY4LjQ4NzEyOTIxMTQyNTc4LCJoZWlnaHQiOjEuOTA1MDA1MDk3Mzg5MjIxMn19XX0sImNodW5rSWQiOiI3OTc4Zjg1YS1iYTEyLTQ2ZTYtYjZkMC0yMmM2MGIwODlkNTciLCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI5MCJ9fV0%3D.

Table 4: Socio-Economic Impacts of Harmful Algal Blooms

| Sector | Impacts |
| --- | --- |
| Fisheries | Decline in fish yields, mass mortalities, gear damage, market devaluation |
| Agriculture | Crop contamination, soil degradation, livestock losses |
| Tourism | Deterrence of visitors, infrastructure decay, reputational damage |
| Public Health | Gastrointestinal illnesses, liver damage, neurological effects, carcinogenicity |

10. Case Studies: Severe HAB Events in Major Indian Lakes

Bellandur Lake in Bengaluru exemplifies the catastrophic frothing crises driven by untreated sewage and industrial effluents, leading to prolific cyanobacterial blooms dominated by Microcystis spp. During peak events in 2015-2017, massive foam plumes comprising surfactants, proteins, and extracellular polysaccharides engulfed 80% of the 3.5 km² surface, releasing volatile organic compounds and hydrogen sulfide odors that permeated urban neighborhoods (Nair & Nayak, 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hdnlhIFZpa3JhbWFuIiwiZmFtaWx5IjoiTmFpciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQcmF0ZWVwIEt1bWFyIiwiZmFtaWx5IjoiTmF5YWsifX1dLCJpZHMiOnsiaXNzbiI6WyIyMjk2LTc3NDUiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzODkvZm1hcnMuMjAyMy4xMDg3Mjk2Iiwib3BlbmFsZXgiOiJXNDM4MjcyMzEzMyJ9LCJ0aXRsZSI6IlVuY292ZXJpbmcgd2F0ZXIgcXVhbGl0eSBhbmQgZXZhbHVhdGluZyB2dWxuZXJhYmlsaXRpZXMgb2Ygc21hbGwtc2NhbGUgZmlzaGVyaWVzIGluIENoaWxpa2EgTGFnb29uLCBJbmRpYSIsImFic3RyYWN0IjoiU21hbGwtc2NhbGUgZmlzaGVyaWVzIChTU0ZzKSBwbGF5IGEgc3Ryb25nIHJvbGUgaW4gc3VzdGFpbmluZyBtaWxsaW9ucyBvZiBsaXZlbGlob29kcywgZm9vZCBzZWN1cml0eSwgbnV0cml0aW9uLCBhbmQgaW5jb21lIGdsb2JhbGx5IGJ1dCB0aGUgZmlzaGVycyBlbmdhZ2VkIGluIHRoaXMgc2VjdG9yIHNpbXVsdGFuZW91c2x5IGV4cGVyaWVuY2UgaGlnaCBsZXZlbHMgb2YgdnVsbmVyYWJpbGl0eSBhbmQgcHJvY2Vzc2VzIG9mIG1hcmdpbmFsaXNhdGlvbi4gU2V2ZXJhbCBmYWN0b3JzIGFyZSBhdHRyaWJ1dGVkIHRvIHRoZSBtdWx0aWRpbWVuc2lvbmFsIHZ1bG5lcmFiaWxpdGllcyB0aGUgc21hbGwtc2NhbGUgZmlzaGVycyBleHBlcmllbmNlLCBzcGFubmluZyBib3RoIG5hdHVyYWwgKGUuZy4sIG5hdHVyYWwgZGlzYXN0ZXJzLCBlY29zeXN0ZW0gY2hhbmdlKSBhbmQgYW50aHJvcG9nZW5pYyAoZS5nLiwgcG9saWN5IGNoYW5nZSwgaHlkcm9sb2dpY2FsIGludGVydmVudGlvbnMsIGFxdWFjdWx0dXJlKSBwcmVzc3VyZXMuIFdoaWxlIHRoZXJlIGlzIG11Y2ggbGl0ZXJhdHVyZSBvbiB2YXJpb3VzIG5hdHVyYWwgYW5kIGh1bWFuIGRyaXZlcnMgb2YgdnVsbmVyYWJpbGl0eSBpbiBzbWFsbC1zY2FsZSBmaXNoZXJ5IGNvbW11bml0aWVzLCBhbiBhYnNlbmNlIG9mIHJlc2VhcmNoIGNvbm5lY3RpbmcgdnVsbmVyYWJpbGl0eSB3aXRoIHdhdGVyIHF1YWxpdHkgaXMgZXZpZGVudC4gRmlzaGVyIGNvbW11bml0aWVzIG9mdGVuIHRhbGsgYWJvdXQgZmlzaCBpbiByZWxhdGlvbiB0byB0aGUgaGVhbHRoIG9mIHRoZWlyIGFxdWF0aWMgaGFiaXRhdHMgd2hlcmVpbiB3YXRlciBxdWFsaXR5IGlzIHNlZW4gYXMgYSBrZXkgcGFyYW1ldGVyLiBUaGUgbGluayBiZXR3ZWVuIGhlYWx0aHkgZmlzaCBhbmQgZ29vZCBxdWFsaXR5IHdhdGVyIGhhcyBzaWduaWZpY2FudCBpbXBsaWNhdGlvbnMgZm9yIHN0cm9uZyBhbmQgdmlhYmxlIGZpc2hpbmcgY29tbXVuaXRpZXMuIFRoaXMgcGFwZXIgZXhhbWluZXMgdGhlc2UgbGlua3MgZnVydGhlciBieSBmb2N1c2luZyBvbiB0aGUgbmF0dXJlIG9mIHZ1bG5lcmFiaWxpdGllcyBjYXVzZWQgYnkgd2F0ZXIgcXVhbGl0eSBjaGFuZ2VzIGluIHRoZSBzbWFsbC1zY2FsZSBmaXNoZXJ5IHN5c3RlbSBvZiBDaGlsaWthIExhZ29vbiBpbiBJbmRpYS4gV2UgdW5kZXJ0YWtlIGRldGFpbGVkIGFuYWx5c2lzIG9mIHRoZSBpbnZhc2l2ZSBzaHJpbXAgYXF1YWN1bHR1cmUgYWN0aXZpdGllcyBhbmQgaHlkcm9sb2dpY2FsIGludGVydmVudGlvbnMgZm9yIG9wZW5pbmcgb2YgYSBsYWdvb24gaW5sZXQgd2l0aCB0aGUgQmF5IG9mIEJlbmdhbCBhcyB0d28gZG9taW5hbnQgZHJpdmVycyBhZHZlcnNlbHkgaW1wYWN0aW5nIHdhdGVyIHF1YWxpdHkgYW5kIGluY3JlYXNpbmcgdnVsbmVyYWJpbGl0aWVzIG9mIHRoZSBlbnRpcmUgc21hbGwtc2NhbGUgZmlzaGVyaWVzIHNvY2lhbC1lY29sb2dpY2FsIHN5c3RlbS4gT3VyIGFuYWx5c2lzIHN1Z2dlc3RzIHRoYXQgdGhlcmUgYXJlIHN0cm9uZyBpbnRlcmNvbm5lY3Rpb25zIGJldHdlZW4gY2hhbmdlcyBpbiB3YXRlciBxdWFsaXR5IGFuZCB0aGUgbGV2ZWxzIG9mIHZ1bG5lcmFiaWxpdGllcyBpbiB0aGUgU1NGcyBvZiBDaGlsaWthIExhZ29vbi4gUG9sbHV0YW50cyBzdWNoIGFzIHBlc3RpY2lkZXMsIGFuZCBvcmdhbmljIGNvbXBvdW5kcyBhY2N1bXVsYXRlIGluIGZpc2ggdGlzc3VlcyBhbmQgYWZmZWN0IHRoZWlyIGdyb3d0aCwgcmVwcm9kdWN0aW9uLCBhbmQgb3ZlcmFsbCBoZWFsdGguIFRoaXMgbGVkIHRvIGRlY2xpbmVzIGluIGZpc2ggcG9wdWxhdGlvbnMsIG1ha2luZyBpdCBtb3JlIGRpZmZpY3VsdCBmb3IgZmlzaGVycyB0byBtYWtlIGEgbGl2aW5nLiBJbiBhZGRpdGlvbiB0byBkaXJlY3QgaW1wYWN0cyBvbiBmaXNoIHBvcHVsYXRpb25zLCBwb29yIHdhdGVyIHF1YWxpdHkgYWxzbyBoYXMgaW5kaXJlY3QgZWZmZWN0cyBvbiB0aGUgc29jaWFsIGFuZCBlY29ub21pYyB2dWxuZXJhYmlsaXR5IG9mIFNTRnMuIEZvciBleGFtcGxlLCBjb250YW1pbmF0aW9uIG9mIHdhdGVyIHNvdXJjZXMgbGVkIHRvIHRoZSByZWR1Y2VkIG51bWJlciBvZiBmaXNoIHNwZWNpZXMgcmVkdWNpbmcgdGhlIGFtb3VudCBvZiB0aW1lIGZpc2hlcnMgY2FuIHNwZW5kIG9uIGZpc2hpbmcgYWN0aXZpdGllcy4gVGhpcyBhbHNvIGFmZmVjdGVkIHRoZSBtYXJrZXRhYmlsaXR5IG9mIGZpc2ggcHJvZHVjdHMsIHJlZHVjZWQgaW5jb21lIGFuZCBpbmNyZWFzZWQgcG92ZXJ0eS4gVG8gZnVsbHkgdW5kZXJzdGFuZCB0aGUgaW50ZXJjb25uZWN0aW9ucyBiZXR3ZWVuIHdhdGVyIHF1YWxpdHkgYW5kIHZ1bG5lcmFiaWxpdHkgaW4gU1NGcyBpbiBDaGlsaWthIExhZ29vbiwgaXQgaXMgaW1wb3J0YW50IHRvIGNvbnNpZGVyIGJvdGggZW52aXJvbm1lbnRhbCBhbmQgc29jaWFsIGZhY3RvcnMsIGFzIHdlbGwgYXMgdGhlIGNvbXBsZXggZmVlZGJhY2sgbG9vcHMgYmV0d2VlbiB0aGVzZSBmYWN0b3JzLiBUaGUgc3R1ZHkgaGVscHMgaW4gYnJpZGdpbmcgYSBjcnVjaWFsIGdhcCBpbiBvdXIgdW5kZXJzdGFuZGluZyBvZiB0aGUgcm9sZSBvZiB3YXRlciBxdWFsaXR5IGluIHZ1bG5lcmFiaWxpdHkgYW5hbHlzaXMgd2l0aGluIHJlc291cmNlIGRlcGVuZGVudCBjb21tdW5pdGllcy4gV2UgY29uY2x1ZGUgd2l0aCBrZXkgaW5zaWdodHMgb24gcG9zc2libGUgY29waW5nIHJlc3BvbnNlcyBhbmQgYWRhcHRpdmUgY2FwYWNpdHkgbmVjZXNzYXJ5IGZvciB0aGUgc21hbGwtc2NhbGUgZmlzaGVyaWVzIGNvbW11bml0aWVzIHRvIHRyYW5zaXRpb24gdG93YXJkIHZpYWJpbGl0eS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMzM4OS9mbWFycy4yMDIzLjEwODcyOTYiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vd3d3LmZyb250aWVyc2luLm9yZy9hcnRpY2xlcy8xMC4zMzg5L2ZtYXJzLjIwMjMuMTA4NzI5Ni9wZGYiLCJpc3N1ZWREYXRlIjoiMjAyMy0wNi0yOSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZyb250aWVycyBpbiBNYXJpbmUgU2NpZW5jZSIsInZvbHVtZSI6MTAsImltcGFjdEZhY3RvciI6IjQuMjgifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRnJvbnRpZXJzIE1lZGlhIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo1MSwiY2l0ZWRCeUNvdW50Ijo1fX0sInF1b3RlIjoiV2F0ZXIgcXVhbGl0eSBvZiBDaGlsaWthIExhZ29vbiBoYXMgZGVncmFkZWQgZHVlIHRvIGFudGhyb3BvZ2VuaWMgaW50ZXJ2ZW50aW9ucyBhbmQgbmF0dXJhbCBmb3JjZXMgb2YgY2hhbmdlLiBWYXJpb3VzIGFjdGl2aXRpZXMgaW4gdGhlIENoaWxpa2EgTGFnb29uLCBzdWNoIGFzIGluZHVzdHJpYWwgd2FzdGV3YXRlciBkaXNwb3NhbCwgc2V3YWdlIGR1bXBpbmcsIGFxdWFjdWx0dXJlLCBoeWRyb2xvZ2ljYWwgaW50ZXJ2ZW50aW9ucywgYW5kIGN5Y2xvbmVzLCBhcmUgcHJvZHVjaW5nIHNhbGluaXR5IGNoYW5nZXMsIHNlZGltZW50IGRlcG9zaXRpb24sIG51dHJpZW50IGVucmljaG1lbnQsIGV1dHJvcGhpY2F0aW9uLCBhbmQgZGVhZCB6b25lcywgYXMgc3RhdGVkLiBGaXNoIHN1cnZpdmFsIGlzIGF0IHJpc2sgYXMgYSByZXN1bHQgb2YgdGhlIHNpZ25pZmljYW50IHJlZHVjdGlvbiBpbiBlbnZpcm9ubWVudGFsIGNpcmN1bXN0YW5jZXMsIHdoaWNoIGRpbWluaXNoZXMgZmlzaGVybWVu4oCZcyBpbmNvbWUgYW5kIHB1c2hlcyB0aGVtIG91dCBvZiBzb2NpZXR5LiBUaGVzZSBjYXVzZXMgYWZmZWN0aW5nIHRoZSB3YXRlciBxdWFsaXR5IG9mIHRoZSBMYWdvb24gZGVwaWN0IG1hbnkgZmFjZXMgb2YgdnVsbmVyYWJpbGl0eSBpbiBTU0YgY29tbXVuaXRpZXMuIEFzIHRoZSBDaGlsaWthIExhZ29vbiBlbnZpcm9ubWVudCBpcyBleHBlY3RlZCB0byB1bmRlcmdvIGluY3JlYXNpbmdseSBkcmFzdGljIGNoYW5nZXMgKHN1Y2ggYXMgY2F0Y2ggY29tcG9zaXRpb24sIGNhdGNoIGNhcGFjaXR5LCBiaW9kaXZlcnNpdHkgdmFyaWF0aW9ucywgd2F0ZXIgcXVhbGl0eSwgYW5kIGZpc2hlcnkgcmV2ZW51ZXMpLCBpbnRlcm5hdGlvbmFsIHBvbGljeSBoYXMgYmVjb21lIG1vcmUgaW50ZXJlc3RlZCBpbiBhZGFwdGF0aW9uIG1lYXN1cmVzLiBHaXZlbiBob3cgZGl2ZXJzZSBTU0YgZWNvc3lzdGVtcyBhcmUgZXhwZWN0ZWQgdG8gYmUgYmV0d2VlbiBjb250aW5lbnRzLCB0aGVyZSBpcyBubyDigJxvbmUgc2l6ZSBmaXRzIGFsbOKAnSBhcHByb2FjaC4gVGhlIHBhcGVyIG91dGxpbmVkIHNldmVyYWwgY29waW5nIGFuZCBhZGFwdGF0aW9uIG1lY2hhbmlzbXMgdG8gaW1wcm92ZSB3YXRlciBxdWFsaXR5IGFuZCBjb250aW51ZSBmaXNoaW5nIGluIGEgc3VzdGFpbmFibGUgd2F5LiBBZGFwdGF0aW9uIHN0cmF0ZWdpZXMgY2FuIG9jY2FzaW9uYWxseSBoYXZlIHVuZm9yZXNlZW4gY29uc2VxdWVuY2VzIGZvciBmaXNoZXJpZXMgYW5kIGZpc2hpbmcgY29tbXVuaXRpZXMuIEFueSB1bmV4cGVjdGVkIGNvbnNlcXVlbmNlcyBvZiBhZGFwdGF0aW9uIGNhbiBiZSBtaXRpZ2F0ZWQgdGhyb3VnaCBsb25nLXRlcm0gcGxhbm5pbmcgYW5kIGlkZW50aWZpY2F0aW9uIG9mIGZ1dHVyZSBvcHRpb25zIHRocm91Z2ggc2NlbmFyaW8gYW5hbHlzaXMuIFRoZSBmaW5kaW5ncyBvZiB0aGlzIHBhcGVyIGNhbiBiZSB1dGlsaXplZCB0byBkZXZlbG9wIHBvbGljaWVzIHRvIHByb3RlY3Qgd2F0ZXIgcXVhbGl0eSBhbmQgU1NGcyBpbiBhIHZhcmlldHkgb2YgY29hc3RhbCBsYWdvb24gc2V0dGluZ3MuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjIyLjI4NTAzNjA4NzAzNjEzMywibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ5MzA1MzQzNjI3OTMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjIzLjc3MzUxOTUxNTk5MTIxLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDgzMzY0MTA1MjI0NjEsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjI1LjI1NTU4NDcxNjc5Njg3NSwibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ4OTU1OTE3MzU4Mzk4NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjYuNzQ0MDY4MTQ1NzUxOTUzLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNTA2ODA1NDE5OTIxODc1LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyOC4yMjYxMzMzNDY1NTc2MTcsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40OTA0NjMyNTY4MzU5NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjkuNzE0NjE2Nzc1NTEyNjk1LCJsZWZ0Ijo4LjMzMTU5MjU1OTgxNDQ1Mywid2lkdGgiOjQwLjQ5NDA5ODY2MzMzMDA4LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozMS4xOTY2ODM4ODM2NjY5OTIsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40OTQzOTIzOTUwMTk1MywiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MzIuNjg1MTY1NDA1MjczNDQsImxlZnQiOjguMzMxNTk2Mzc0NTExNzE5LCJ3aWR0aCI6NDAuNDg3NDQ1ODMxMjk4ODMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM0LjE2NzIzMjUxMzQyNzczNCwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40OTc3NDU1MTM5MTYwMTYsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM1LjY1NTcxNTk0MjM4MjgxLCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ5ODYzODE1MzA3NjE3LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozNy4xMzc3ODMwNTA1MzcxMSwibGVmdCI6MTAuNjg1MTg2Mzg2MTA4Mzk4LCJ3aWR0aCI6MzguMTQxMTA1NjUxODU1NDcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM4LjYyNjI2NjQ3OTQ5MjE5LCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4NzE1MjA5OTYwOTM3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDAuMTA4MzMzNTg3NjQ2NDg0LCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4OTg1NjcxOTk3MDcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQxLjU5NjgxMzIwMTkwNDMsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDg4NjY2NTM0NDIzODMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQzLjA3ODg4MDMxMDA1ODU5NCwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40ODE4NjExMTQ1MDE5NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDQuNTY3MzYzNzM5MDEzNjcsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDgzMDU4OTI5NDQzMzYsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQ2LjA0OTQzMDg0NzE2Nzk3LCJsZWZ0Ijo4LjMzMzEwODkwMTk3NzUzOSwid2lkdGgiOjQwLjUwMDc0NzY4MDY2NDA2LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo0Ny41Mzc5MTQyNzYxMjMwNSwibGVmdCI6OC4zMzMxMDg5MDE5Nzc1MzksIndpZHRoIjo0MC40OTU2MDU0Njg3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDkuMDE5OTgxMzg0Mjc3MzQ0LCJsZWZ0Ijo4LjMzMzEwNjA0MDk1NDU5LCJ3aWR0aCI6NDAuNTA2ODE2ODY0MDEzNjcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUwLjUwODQ2NDgxMzIzMjQyLCJsZWZ0Ijo4LjMzMzEwNjA0MDk1NDU5LCJ3aWR0aCI6NDAuNDg4NTA2MzE3MTM4NjcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUxLjk5MDUyODEwNjY4OTQ1LCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDk3NzI2NDQwNDI5NjksImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUzLjQ3OTAxMTUzNTY0NDUzLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6MzMuMTU5NDQ2NzE2MzA4NTk0LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo1NC45NjEwNzg2NDM3OTg4MywibGVmdCI6MTAuNjgzNjcwMDQzOTQ1MzEyLCJ3aWR0aCI6MzguMTM2NTk2Njc5Njg3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NTYuNDQ5NTYyMDcyNzUzOTA2LCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDg3MTQ0NDcwMjE0ODQ0LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEzIn19XQ%3D%3D. Hypoxia from algal respiration caused fish yields to plummet by 60-70%, erasing INR 500-1000 crore in peri-urban fishery revenues annually. Peri-urban agriculture suffered as toxin-laden irrigation bioaccumulated microcystins in vegetables at 5-20 μg/kg, exceeding WHO thresholds and sparking export bans (Gobler et al., 2024; Wang et al., 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkNocmlzdG9waGVyIEouIiwiZmFtaWx5IjoiR29ibGVyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlJ1dGggVy4iLCJmYW1pbHkiOiJEcmlua3dhdGVyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkEuIFcuIiwiZmFtaWx5IjoiQW50aG9ueSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJKZW5uaWZlciBBLiIsImZhbWlseSI6IkdvbGVza2kifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW5uIE1hcmllIEUuIiwiZmFtaWx5IjoiRmFtdWxhcm8tUGVjb3JhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1pY2hhZWwgQi4iLCJmYW1pbHkiOiJXYWxsYWNlIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5vcmEgUi4gVy4iLCJmYW1pbHkiOiJTdHJhcXVhZGluZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJSb25vam95IiwiZmFtaWx5IjoiSGVtIn19XSwiaWRzIjp7Imlzc24iOlsiMTY2NC0zMDJYIl0sImlzYm4iOltdLCJkb2kiOiIxMC4zMzg5L2ZtaWNiLjIwMjQuMTQ2NDY4NiIsInBtaWQiOiIzOTU2ODk5NyIsIm9wZW5hbGV4IjoiVzQ0MDQxMjA5NTIiLCJleGEiOiJodHRwczovL3d3dy5mcm9udGllcnNpbi5vcmcvam91cm5hbHMvbWljcm9iaW9sb2d5L2FydGljbGVzLzEwLjMzODkvZm1pY2IuMjAyNC4xNDY0Njg2L2Z1bGwifSwidGl0bGUiOiJTZXdhZ2UtYW5kIGZlcnRpbGl6ZXItZGVyaXZlZCBudXRyaWVudHMgYWx0ZXIgdGhlIGludGVuc2l0eSwgZGl2ZXJzaXR5LCBhbmQgdG94aWNpdHkgb2YgaGFybWZ1bCBjeWFub2JhY3RlcmlhbCBibG9vbXMgaW4gZXV0cm9waGljIGxha2VzIiwiYWJzdHJhY3QiOiJDeWFub2JhY3RlcmlhbCBoYXJtZnVsIGFsZ2FsIGJsb29tcyAoQ0hBQnMpIGFyZSBwcm9tb3RlZCBieSBleGNlc3NpdmUgbnV0cmllbnQgbG9hZGluZyBhbmQsIHdoaWxlIGZlcnRpbGl6ZXJzIGFuZCBzZXdhZ2UgYXJlIHRoZSBtb3N0IHByZXZhbGVudCBleHRlcm5hbCBudXRyaWVudCBzb3VyY2VzIGluIG1vc3Qgd2F0ZXJzaGVkcywgdGhlIGRpZmZlcmVudGlhbCBlZmZlY3RzIG9mIHRoZXNlIG51dHJpZW50IHNvdXJjZXMgb24gQ0hBQnMgYXJlIHVua25vd24uIEhlcmUsIHdlIHRyYWNrZWQgQ0hBQnMgYW5kIHBlcmZvcm1lZCBleHBlcmltZW50cyBpbiBmaXZlIGRpc3RpbmN0IGxha2VzIGFjcm9zcyB0aGUgTm9ydGhlcm4gVVMgaW5jbHVkaW5nIExha2UgRXJpZS4gRmVydGlsaXplcnMgd2l0aCBhbW1vbml1bSBhbmQgb3J0aG9waG9zcGhhdGUsIG1lbWJyYW5lICgwLjIgzrxtKS1maWx0ZXJlZCBzZXdhZ2UgKGRvbWluYXRlZCBieSByZWR1Y2VkIGZvcm1zIG9mIG5pdHJvZ2VuKSBzYW5kLWFuZCBtZW1icmFuZS1maWx0ZXJlZCBzZXdhZ2UgKGRvbWluYXRlZCBieSBuaXRyYXRlKSwgYW5kIGFuIGlub3JnYW5pYyBudXRyaWVudCBzb2x1dGlvbiBvZiBhbW1vbml1bSBhbmQgb3J0aG9waG9zcGhhdGUgd2VyZSB1c2VkIGFzIGV4cGVyaW1lbnRhbCBudXRyaWVudCBzb3VyY2VzIGZvciBDSEFCcyBhdCBOLWVxdWl2YWxlbnQsIGVudmlyb25tZW50YWxseSByZWFsaXN0aWMgY29uY2VudHJhdGlvbnMuIFBoeXRvcGxhbmt0b24gY29tbXVuaXRpZXMgd2VyZSBldmFsdWF0ZWQgZmx1b3JvbWV0cmljYWxseSwgbWljcm9zY29waWNhbGx5LCBhbmQgdmlhIGhpZ2ggdGhyb3VnaHB1dCBzZXF1ZW5jaW5nIG9mIHRoZSAxNlMgclJOQSBnZW5lLCBhbmQgbGV2ZWxzIG9mIG1pY3JvY3lzdGluIGFuZCB0aGUgzrQgMTUgTiBjb250ZW50IG9mIHBhcnRpY3VsYXRlIG9yZ2FuaWMgbml0cm9nZW4gKM60UE8gMTUgTikgd2VyZSBxdWFudGlmaWVkLiBGZXJ0aWxpemVyIGFuZCBib3RoIHNvdXJjZXMgb2Ygd2FzdGV3YXRlciBpbmNyZWFzZWQgdGhlIGFidW5kYW5jZSBvZiBjeWFub2JhY3RlcmlhIGluIGFsbCBleHBlcmltZW50cyBhY3Jvc3MgYWxsIGZpdmUgbGFrZXMgKCBwICZhbXA7bHQ7IDAuMDUgZm9yIGFsbCkgd2hlcmVhcyBlZmZlY3RzIG9uIGV1a2FyeW90aWMgcGh5dG9wbGFua3RvbiB3ZXJlIGxpbWl0ZWQuIFNhbmQtZmlsdGVyZWQgc2V3YWdlIGNvbnRhaW5lZCBsZXNzIFAsIG9yZ2FuaWMgbWF0dGVyLCBhbmQgYW1tb25pdW0gYnV0IG1vcmUgbml0cmF0ZSBhbmQgaGFkIGEgMjUlIGxlc3MgcG90ZW50IHN0aW11bGF0b3J5IGVmZmVjdCBvbiBjeWFub2JhY3RlcmlhIHRoYW4gbWVtYnJhbmUtZmlsdGVyZWQgc2V3YWdlLCBzdWdnZXN0aW5nIG5pdHJpZmljYXRpb24gbWF5IHBsYXkgYSByb2xlIGluIHJlZHVjaW5nIENIQUJzLiBGZXJ0aWxpemVyIGluY3JlYXNlZCBtaWNyb2N5c3RpbiBsZXZlbHMgYW5kIGRlY3JlYXNlZCB0aGUgzrRQTyAxNSBOIHdoZXJlYXMgd2FzdGV3YXRlciBpbmNyZWFzZWQgzrRQTyAxNSBOICggcCAmYW1wO2x0OyAwLjA1IGZvciBhbGwpLiBNaWNyb2N5c3RpcyB3YXMgdGhlIGdlbnVzIG1vc3QgY29uc2lzdGVudGx5IHByb21vdGVkIGJ5IG51dHJpZW50IHNvdXJjZXMgKCBwICZhbXA7bHQ7IDAuMDUgaW4gYWxsIGV4cGVyaW1lbnRzKSwgZm9sbG93ZWQgYnkgQ3lhbm9iaXVtICggcCAmYW1wO2x0OyAwLjA1IGluIDUwJSBvZiBleHBlcmltZW50cyksIHdpdGggaW5jcmVhc2VzIGluIE1pY3JvY3lzdGlzIGJpb21hc3MgY29uc2lzdGVudGx5IGVsaWNpdGVkIGJ5IG1lbWJyYW5lLWZpbHRlcmVkIHdhc3Rld2F0ZXIuIENvbGxlY3RpdmVseSwgcmVzdWx0cyBkZW1vbnN0cmF0ZSB0aGF0IGRpZmZlcmluZyB0eXBlcyBvZiBzZXdhZ2UgZGlzY2hhcmdlIGFuZCBmZXJ0aWxpemVycyBjYW4gcHJvbW90ZSBDSEFCIGludGVuc2l0eSBhbmQgdG94aWNpdHksIHdoaWxlIGNvbmN1cnJlbnRseSBhbHRlcmluZyBDSEFCIGRpdmVyc2l0eSBhbmQgzrRQTyAxNSBOLiBXaGlsZSBtZW1icmFuZS1maWx0ZXJlZCBzZXdhZ2UgY29uc2lzdGVudGx5IGZhdm9yZWQgTWljcm9jeXN0aXMgLCB0aGUgZGlzY2hhcmdlIG9mIHNld2FnZSB0aHJvdWdoIHNhbmRzIG11dGVkIGJsb29tIGludGVuc2l0eSBzdWdnZXN0aW5nIHNhbmQtYmVkcyBtYXkgcmVwcmVzZW50IGEgdG9vbCB0byByZW1vdmUga2V5IG51dHJpZW50cyBhbmQgcGFydGlhbGx5IG1pdGlnYXRlIENIQUJzLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzg5L2ZtaWNiLjIwMjQuMTQ2NDY4NiIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cuZnJvbnRpZXJzaW4ub3JnL2pvdXJuYWxzL21pY3JvYmlvbG9neS9hcnRpY2xlcy8xMC4zMzg5L2ZtaWNiLjIwMjQuMTQ2NDY4Ni9wZGYiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMS0wNiIsImFjY2Vzc2VkRGF0ZSI6IjIwMjYtMDIiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZyb250aWVycyBpbiBNaWNyb2Jpb2xvZ3kiLCJ2b2x1bWUiOjE1LCJpbXBhY3RGYWN0b3IiOiIzLjk5In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTQ2NDY4NiwibGFzdFBhZ2UiOjE0NjQ2ODZ9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRnJvbnRpZXJzIE1lZGlhIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo3MywiY2l0ZWRCeUNvdW50Ijo5fX0sInF1b3RlIjoiZGVyaXZlZCBudXRyaWVudHMgYWx0ZXIgdGhlIGludGVuc2l0eSwgZGl2ZXJzaXR5LCBhbmQgdG94aWNpdHkgb2YgaGFybWZ1bCBjeWFub2JhY3RlcmlhbCBibG9vbXMgLi4uIEN5YW5vYmFjdGVyaWFsIGhhcm1mdWwgYWxnYWwgYmxvb21zIChDSEFCcykgYXJlIHByb21vdGVkIGJ5IGV4Y2Vzc2l2ZSBudXRyaWVudCBsb2FkaW5nIGFuZCwgd2hpbGUgZmVydGlsaXplcnMgYW5kIHNld2FnZSBhcmUgdGhlIG1vc3QgcHJldmFsZW50IGV4dGVybmFsIG51dHJpZW50IHNvdXJjZXMgaW4gbW9zdCB3YXRlcnNoZWRzLCB0aGUgZGlmZmVyZW50aWFsIGVmZmVjdHMgb2YgdGhlc2UgbnV0cmllbnQgLi4uIG9yZ2FuaWMgbml0cm9nZW4gKM60UE8xNU4pIHdlcmUgcXVhbnRpZmllZC4gRmVydGlsaXplciBhbmQgYm90aCBzb3VyY2VzIG9mIHdhc3Rld2F0ZXIgaW5jcmVhc2VkIHRoZSBhYnVuZGFuY2Ugb2YgLi4uIHBoeXRvcGxhbmt0b24gd2VyZSBsaW1pdGVkLiBTYW5kLWZpbHRlcmVkIHNld2FnZSBjb250YWluZWQgbGVzcyBQLCBvcmdhbmljIG1hdHRlciwgYW5kIGFtbW9uaXVtIGJ1dCBtb3JlIG5pdHJhdGUgYW5kIC4uLiBkaWZmZXJpbmcgdHlwZXMgb2Ygc2V3YWdlIGRpc2NoYXJnZSBhbmQgZmVydGlsaXplcnMgY2FuIHByb21vdGUgQ0hBQiBpbnRlbnNpdHkgYW5kIHRveGljaXR5LCB3aGlsZSBjb25jdXJyZW50bHkgLi4uIHNld2FnZSB0aHJvdWdoIHNhbmRzIG11dGVkIGJsb29tIGludGVuc2l0eSBzdWdnZXN0aW5nIHNhbmQtYmVkcyBtYXkgcmVwcmVzZW50IGEgdG9vbCB0byByZW1vdmUga2V5IG51dHJpZW50cyBhbmQgLi4uICggV2F0c29uIGV0IGFsLiwgMjAxNiA7IEJ1bGxlcmphaG4gZXQgYWwuLCAyMDE2IDsgSHVpc21hbiBldCBhbC4sIDIwMTggKSBhbmQgZXhjZXNzaXZlIG51dHJpZW50IGxvYWRpbmcgaXMgdGhlIHByb2Nlc3MgbW9zdCBjb21tb25seSBjaXRlZCBhcyB0aGUgZW52aXJvbm1lbnRhbCBmYWN0b3IgcHJvbW90aW5nIENIQUJzICgifSx7Im9yaWdpbiI6NCwid29yayI6eyJ0eXBlIjo4LCJzdWJ0eXBlIjp7Im9uZW9mS2luZCI6ImFydGljbGVUeXBlIiwiYXJ0aWNsZVR5cGUiOjF9LCJjb250cmlidXRvcnMiOlt7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJYaXdlbiIsImZhbWlseSI6IldhbmcifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiS3VuIiwiZmFtaWx5IjoiU2hpIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ill1bmxpbiIsImZhbWlseSI6IlpoYW5nIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkJvcWlhbmciLCJmYW1pbHkiOiJRaW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiWWlibyIsImZhbWlseSI6IlpoYW5nIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IldlaWppYSIsImZhbWlseSI6IldhbmcifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUi4gSWVzdHluIiwiZmFtaWx5IjoiV29vbHdheSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTaGlsb25nIiwiZmFtaWx5IjoiUGlhbyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJFcmlrIiwiZmFtaWx5IjoiSmVwcGVzZW4ifX1dLCJpZHMiOnsiaXNzbiI6W10sImlzYm4iOltdLCJkb2kiOiIxMC4yMTIwMy9ycy4zLnJzLTI0NDIxNDIvdjEiLCJvcGVuYWxleCI6Ilc0MzE3MDM4ODI3In0sInRpdGxlIjoiTGFrZSB3YXJtaW5nIGFuZCBpbmNyZWFzZWQgaGVhdHdhdmVzIHJldmVhbGVkIGJ5IGEgbm92ZWwgZGF0YS1kcml2ZW4gbW9kZWxpbmcgYXBwcm9hY2giLCJhYnN0cmFjdCI6Ijx0aXRsZT5BYnN0cmFjdDwvdGl0bGU%2BIENsaW1hdGUgY2hhbmdlIGNvdWxkIHNlcmlvdXNseSB0aHJlYXRlbiBnbG9iYWwgbGFrZSBlY29zeXN0ZW1zIGJ5IHdhcm1pbmcgbGFrZSBzdXJmYWNlIHdhdGVyIGFuZCBpbmNyZWFzaW5nIHRoZSBvY2N1cnJlbmNlIG9mIGxha2UgaGVhdHdhdmVzLiBZZXQsIHRoZXJlIGFyZSBncmVhdCB1bmNlcnRhaW50aWVzIGluIHF1YW50aWZ5aW5nIGxha2UgdGVtcGVyYXR1cmUgY2hhbmdlcyBnbG9iYWxseSBkdWUgdG8gbGFjayBvZiBhY2N1cmF0ZSBsYXJnZS1zY2FsZSBtb2RlbCBzaW11bGF0aW9ucy4gSGVyZSwgd2UgaW50ZWdyYXRlZCBzYXRlbGxpdGUgb2JzZXJ2YXRpb25zIGFuZCBhIG51bWVyaWNhbCBtb2RlbCB0byBpbXByb3ZlIGxha2UgdGVtcGVyYXR1cmUgbW9kZWxsaW5nIGFuZCB0byBleHBsb3JlIHRoZSBtdWx0aWZhY2V0ZWQgY2hhcmFjdGVyaXN0aWNzIG9mIHRyZW5kcyBpbiBzdXJmYWNlIHRlbXBlcmF0dXJlcyBhbmQgbGFrZSBoZWF0d2F2ZSBvY2N1cnJlbmNlIGluIENoaW5lc2UgbGFrZXMgZnJvbSAxOTgwIHRvIDIxMDAuIE91ciBtb2RlbC1kYXRhIGludGVncmF0aW9uIGFwcHJvYWNoIHJldmVhbGVkIHRoYXQgdGhlIGxha2Ugc3VyZmFjZSB3YXRlcnMgaGF2ZSB3YXJtZWQgYXQgYSByYXRlIG9mIDAuMTEgwrBDIGRlY2FkZS0xIGR1cmluZyB0aGUgcGVyaW9kIDE5ODAtMjAyMSwgYmVpbmcgb25seSBoYWxmIG9mIHRoZSBwdXJlIG1vZGVsLWJhc2VkIGVzdGltYXRlLiBNb3Jlb3Zlciwgb3VyIGFuYWx5c2lzIHN1Z2dlc3RlZCB0aGF0IGFuIGFzeW1tZXRyaWMgc2Vhc29uYWwgd2FybWluZyByYXRlIGhhcyBsZWQgdG8gYSByZWR1Y2VkIHRlbXBlcmF0dXJlIHNlYXNvbmFsaXR5IGluIGVhc3Rlcm4gcGxhaW4gbGFrZXMgYnV0IGFuIGFtcGxpZmllZCBvbmUgaW4gYWxwaW5lIGxha2VzLiBUaGUgZHVyYXRpb25zIG9mIGxha2UgaGVhdHdhdmVzIGhhdmUgYWxzbyBpbmNyZWFzZWQgYXQgYSByYXRlIG9mIDcuNyBkYXlzIGRlY2FkZS0xLiBVbmRlciB0aGUgaGlnaC1ncmVlbmhvdXNlLWdhcy1lbWlzc2lvbiBzY2VuYXJpbywgbGFrZSBzdXJmYWNlIHRlbXBlcmF0dXJlIGFuZCBsYWtlIGhlYXR3YXZlIGR1cmF0aW9uIHdlcmUgcHJvamVjdGVkIHRvIGluY3JlYXNlIGJ5IDIuMiDCsEMgYW5kIDE5NyBkYXlzIGF0IHRoZSBlbmQgb2YgMjFzdCBjZW50dXJ5LCByZXNwZWN0aXZlbHkuIFN1Y2ggZHJhc3RpYyBjaGFuZ2VzIHdvdWxkIHdvcnNlbiB0aGUgZW52aXJvbm1lbnRhbCBjb25kaXRpb25zIG9mIGxha2VzIHN1YmplY3RlZCB0byBoaWdoIGFuZCBpbmNyZWFzaW5nIGFudGhyb3BvZ2VuaWMgcHJlc3N1cmVzLCBwb3NpbmcgZ3JlYXQgdGhyZWF0cyB0byBhcXVhdGljIGJpb2RpdmVyc2l0eSBhbmQgaHVtYW4gaGVhbHRoLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4yMTIwMy9ycy4zLnJzLTI0NDIxNDIvdjEiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vd3d3LnJlc2VhcmNoc3F1YXJlLmNvbS9hcnRpY2xlL3JzLTI0NDIxNDIvbGF0ZXN0LnBkZiIsImlzc3VlZERhdGUiOiIyMDIzLTAxLTE3IiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiUmVzZWFyY2ggU3F1YXJlIChSZXNlYXJjaCBTcXVhcmUpIiwiaW1wYWN0RmFjdG9yIjoiMC40MCJ9LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJSZXNlYXJjaCBTcXVhcmUgKFVuaXRlZCBTdGF0ZXMpIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo1NywiY2l0ZWRCeUNvdW50IjozfX0sInF1b3RlIjoiSWYgdGhlIGJvdHRvbSB3YXRlciB0ZW1wZXJhdHVyZSBhbHNvIGluY3JlYXNlcywgbWluZXJhbGl6YXRpb24gb2Ygb3JnYW5pYyBtYXR0ZXIgd2lsbCBiZSBhY2NlbGVyYXRlZCwgd2hpY2ggd2lsbCBsZWFkIHRvIGhpZ2hlciBlbWlzc2lvbiBvZiBjYXJib24gZGlveGlkZSBhbmQgbWV0aGFuZTQ2LDQ3LCByZWluZm9yY2luZyBjbGltYXRlIHdhcm1pbmcgZHVlIHRvIGdyZWVuaG91c2UgZ2FzZXMuIFRoaXMgcHJvY2VzcyB3aWxsIHNwZWVkIHVwIHRoZSByZWN5Y2xpbmcgb2YgbnV0cmllbnRzIGFuZCBmYXZvciBwaHl0b3BsYW5rdG9uIGdyb3d0aCwgc3VnZ2VzdGluZyBwb3NpdGl2ZSBmZWVkYmFjayBiZXR3ZWVuIGNsaW1hdGUgY2hhbmdlIGFuZCBldXRyb3BoaWNhdGlvbjQ4LiBUb2dldGhlciB3aXRoIHRoZSBoZWF2eSBleHRlcm5hbCBudXRyaWVudCBsb2FkaW5nIGZyb20gZG9tZXN0aWMsIGFncmljdWx0dXJhbCwgYW5kIGluZHVzdHJpYWwgd2FzdGUsIHRoZSBncm93dGggb2YgcGh5dG9wbGFua3RvbiBpcyBwcm9tb3RlZCBieSBob3Qgd2F0ZXIgdW5kZXIgbnV0cmllbnQtcmljaCBjb25kaXRpb25zNDkuIEJlc2lkZXMgdGhhdCwgc3Ryb25nZXIgc3RyYXRp77%2B9Y2F0aW9uIHdpbGwgcGVybWl0IHBoeXRvcGxhbmt0b24gdG8gY2lyY3VsYXRlIGluIHRoZSBldXBob3RpYyB6b25lNTAuIEluY3JlYXNpbmcgYWxnYWUgYmxvb21zIGhhdmUgYmVlbiByZXBvcnRlZCB3b3JsZHdpZGUsIGVzcGVjaWFsbHkgaW4gcmVnaW9ucyB0aGF0IGRlcGVuZCBoaWdobHkgb24gZmVydGlsaXplcnM1MS4gSW4gQ2hpbmEsIHRoZSBvY2N1cnJlbmNlIG9mIGFsZ2FlIGJsb29tcyBpbiB0aGUgMjAxMHMgaGFzIGluY3JlYXNlZCBuZWFybHkgbmluZSB0aW1lcyByZWxhdGl2ZSB0byB0aGUgMTk4MHMtMTk5MHM1MSwgYW5kIG1hbnkgYmxvb21zIGhhdmUgYmVlbiByZXBvcnRlZCByZWNlbnRseSBpbiBuYXR1cmFsIGxha2VzIGFuZCByZXNlcnZvaXJzIChTdXBwbGVtZW50YXJ5IEZpZy4gNykuIEZ1cnRoZXJtb3JlLCB0aGUgd2FybWVyIHN1cmZhY2Ugd2F0ZXIgYW5kIHN0cm9uZ2VyIHN0cmF0ae%2B%2FvWNhdGlvbiB3aWxsIGZhdm9yIHRoZSBkb21pbmFuY2Ugb2YgY3lhbm9iYWN0ZXJpYSBpbiBsYWtlcyBnaXZlbiBpdHMgaGlnaGVyIG9wdGltYWwgZ3Jvd3RoIHRlbXBlcmF0dXJlIGFuZCBhYmlsaXR5IHRvIGFkanVzdCBidW95YW5jeTQ5LCBsZWFkaW5nIHRvIGhpZ2hlciBmcmVxdWVuY3kgb2YgaGFybWZ1bCBhbGdhZSBibG9vbXM0Ny4gVGhlIHRveGljIHN1YnN0YW5jZXMgcHJvZHVjZWQgYnkgY3lhbm9iYWN0ZXJpYSB3aWxsIHRocmVhdGVuIHRoZSBzZWN1cml0eSBvZiBkcmlua2luZyB3YXRlciBhbmQgaHVtYW4gaGVhbHRoNiw1Mi4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo0NS4xMjY5NjA3NTQzOTQ1MywibGVmdCI6NC41NzUxNjI4ODc1NzMyNDIsIndpZHRoIjo5MC45NDEyMDAyNTYzNDc2NiwiaGVpZ2h0IjoxLjc3NzE2MzM4NjM0NDkwOTd9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo0Ny40MDE3MjE5NTQzNDU3LCJsZWZ0Ijo0LjU3NTE2Mjg4NzU3MzI0Miwid2lkdGgiOjkwLjIwNTI0NTk3MTY3OTY5LCJoZWlnaHQiOjEuNzc3MTYzMzg2MzQ0OTA5N319LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjQ5LjY0OTExNjUxNjExMzI4LCJsZWZ0Ijo0LjU3NTE2Mjg4NzU3MzI0Miwid2lkdGgiOjkxLjA0MzczMTY4OTQ1MzEyLCJoZWlnaHQiOjQuMjY4ODY5ODc2ODYxNTcyfX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NTQuMzg4MjE3OTI2MDI1MzksImxlZnQiOjQuNTc1MTYyODg3NTczMjQyLCJ3aWR0aCI6NDguNDc2MjI2ODA2NjQwNjI1LCJoZWlnaHQiOjEuOTk0MTA4MDgwODYzOTUyNn19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjU4LjM5NjQzNDc4MzkzNTU1LCJsZWZ0Ijo0LjU3NTE2Mjg4NzU3MzI0Miwid2lkdGgiOjg3Ljg4MjM4NTI1MzkwNjI1LCJoZWlnaHQiOjEuNzc3MTYzMzg2MzQ0OTA5N319LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjYwLjY0MzgyOTM0NTcwMzEyNSwibGVmdCI6NC41NzUxNjI4ODc1NzMyNDIsIndpZHRoIjo5MC40NTA1NjE1MjM0Mzc1LCJoZWlnaHQiOjEuOTk0MTA4MDgwODYzOTUyNn19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjYzLjEwODE2OTU1NTY2NDA2LCJsZWZ0Ijo0LjU3NTE2Mjg4NzU3MzI0Miwid2lkdGgiOjg5Ljk2NzU5Nzk2MTQyNTc4LCJoZWlnaHQiOjEuOTk0MTA4MDgwODYzOTUyNn19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjY1LjU3MjQ0ODczMDQ2ODc1LCJsZWZ0Ijo0LjU3NTE2Mjg4NzU3MzI0Miwid2lkdGgiOjkwLjU4MDg4Njg0MDgyMDMxLCJoZWlnaHQiOjEuOTk0MTY5NzEyMDY2NjUwNH19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjY4LjAzNjc4ODk0MDQyOTY5LCJsZWZ0Ijo0LjU3NTE2Mjg4NzU3MzI0Miwid2lkdGgiOjg5LjE5MDQzNzMxNjg5NDUzLCJoZWlnaHQiOjYuNTQzNjkzMDY1NjQzMzEwNX19LHsicGFnZU51bWJlciI6NywiYmJveCI6eyJ0b3AiOjc1LjA1MDY1MTU1MDI5Mjk3LCJsZWZ0Ijo0LjU3NTE2Mjg4NzU3MzI0Miwid2lkdGgiOjkwLjM0NzA2ODc4NjYyMTEsImhlaWdodCI6MS45OTQxNjk3MTIwNjY2NTA0fX0seyJwYWdlTnVtYmVyIjo3LCJiYm94Ijp7InRvcCI6NzcuNTE0OTkxNzYwMjUzOSwibGVmdCI6NC41NzUxNjI4ODc1NzMyNDIsIndpZHRoIjo5MC4yMjI0ODg0MDMzMjAzMSwiaGVpZ2h0IjoxLjk5NDE2OTcxMjA2NjY1MDR9fSx7InBhZ2VOdW1iZXIiOjcsImJib3giOnsidG9wIjo3OS45NzkzMzk1OTk2MDkzOCwibGVmdCI6NC41NzUxNjI4ODc1NzMyNDIsIndpZHRoIjo4NS45ODIxMzE5NTgwMDc4MSwiaGVpZ2h0IjoxLjk5NDE2OTcxMjA2NjY1MDR9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI4In19XQ%3D%3D. Cleanup operations, costing millions, highlighted governance lapses, with blooms recurring due to legacy phosphorus in sediments recycling under warming stratification (Jansen et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkpvYWNoaW0iLCJmYW1pbHkiOiJKYW5zZW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiR2F2aW4gTC4iLCJmYW1pbHkiOiJTaW1wc29uIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikdlc2EgQS4iLCJmYW1pbHkiOiJXZXloZW5tZXllciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJMYXVyYSBILiIsImZhbWlseSI6IkjDpHJrw7ZuZW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW5kcmV3IE0uIiwiZmFtaWx5IjoiUGF0ZXJzb24ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUGF1bCBBLiBkZWwiLCJmYW1pbHkiOiJHaW9yZ2lvIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ill2ZXMgVC4iLCJmYW1pbHkiOiJQcmFpcmllIn19XSwiaWRzIjp7Imlzc24iOlsiMTc1OC02NzhYIiwiMTc1OC02Nzk4Il0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDM4L3M0MTU1OC0wMjQtMDIwNTgtMyIsIm9wZW5hbGV4IjoiVzQ0MDAyMDgyMDAiLCJleGEiOiJodHRwczovL3d3dy5uYXR1cmUuY29tL2FydGljbGVzL3M0MTU1OC0wMjQtMDIwNTgtMz9lcnJvcj1jb29raWVzX25vdF9zdXBwb3J0ZWQmY29kZT1hNzRhMjVmZi1hNThkLTRkZjgtOTM2Zi0yNjJlMmUzZTkxOTgifSwidGl0bGUiOiJDbGltYXRlLWRyaXZlbiBkZW94eWdlbmF0aW9uIG9mIG5vcnRoZXJuIGxha2VzIiwiYWJzdHJhY3QiOiJBYnN0cmFjdCBPeHlnZW4gZGVwbGV0aW9uIGNvbnN0aXR1dGVzIGEgbWFqb3IgdGhyZWF0IHRvIGxha2UgZWNvc3lzdGVtcyBhbmQgdGhlIHNlcnZpY2VzIHRoZXkgcHJvdmlkZS4gTW9zdCBvZiB0aGUgd29ybGTigJlzIGxha2VzIGFyZSBsb2NhdGVkICZndDs0NcKwIE4sIHdoZXJlIGFjY2VsZXJhdGVkIGNsaW1hdGUgd2FybWluZyBhbmQgZWxldmF0ZWQgY2FyYm9uIGxvYWRzIG1pZ2h0IHNldmVyZWx5IGluY3JlYXNlIHRoZSByaXNrIG9mIGh5cG94aWEsIGJ1dCB0aGlzIGhhcyBub3QgYmVlbiBzeXN0ZW1hdGljYWxseSBleGFtaW5lZC4gSGVyZSBhbmFseXNpcyBvZiAyLjYgbWlsbGlvbiB3YXRlciBxdWFsaXR5IG9ic2VydmF0aW9ucyBmcm9tIDgsMjg4IGxha2VzIHNob3dzIHRoYXQgYmV0d2VlbiAxOTYwIGFuZCAyMDIyLCBtb3N0IG5vcnRoZXJuIGxha2VzIGV4cGVyaWVuY2VkIHJhcGlkIGRlb3h5Z2VuYXRpb24gc3Ryb25nbHkgbGlua2VkIHRvIGNsaW1hdGUtZHJpdmVuIHByb2xvbmdhdGlvbiBvZiBzdW1tZXIgc3RyYXRpZmljYXRpb24uIE94eWdlbiBsZXZlbHMgZGV0ZXJpb3JhdGVkIG1vc3QgaW4gc21hbGwgbGFrZXMgKCZsdDsxMCBoYSkgb3dpbmcgdG8gdGhlaXIgZ3JlYXRlciB2b2x1bWV0cmljIG94eWdlbiBkZW1hbmQgYW5kIHN1cmZhY2Ugd2FybWluZyByYXRlcywgd2hpbGUgdGhlIGxhcmdlc3QgbGFrZXMgZ2FpbmVkIG94eWdlbiB1bmRlciBtaW5pbWFsIHN0cmF0aWZpY2F0aW9uIGNoYW5nZXMgYW5kIGltcHJvdmVkIGFlcmF0aW9uIGF0IHNwcmluZyBvdmVydHVybnMuIFNlYXNvbmFsIG94eWdlbiBjb25zdW1wdGlvbiByYXRlcyBkZWNsaW5lZCwgZGVzcGl0ZSB3aWRlc3ByZWFkIGJyb3duaW5nLiBQcm9saWZlcmF0aW5nIGFub3hpYSBlbmhhbmNlZCBzZWFzb25hbCBpbnRlcm5hbCBsb2FkaW5nIG9mIEMsIFAgYW5kIE4gYnV0IGRlcGxldGVkIFAgbG9uZy10ZXJtLCBpbmRpY2F0aW5nIHRoYXQgZGVveHlnZW5hdGlvbiBjYW4gZXhoYXVzdCByZWRveC1zZW5zaXRpdmUgZnJhY3Rpb25zIG9mIHNlZGltZW50IG51dHJpZW50IHJlc2Vydm9pcnMuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjEwMzgvczQxNTU4LTAyNC0wMjA1OC0zIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAzOC9zNDE1NTgtMDI0LTAyMDU4LTMiLCJpc3N1ZWREYXRlIjoiMjAyNC0wNy0wMSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTEiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6Ik5hdHVyZSBDbGltYXRlIENoYW5nZSIsImlzc3VlIjo4LCJ2b2x1bWUiOjE0LCJpbXBhY3RGYWN0b3IiOiIxNS4wMiJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjgzMiwibGFzdFBhZ2UiOjgzOH19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJOYXR1cmUgUG9ydGZvbGlvIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo4MCwiY2l0ZWRCeUNvdW50IjozN319LCJxdW90ZSI6IkFlcm9iaWMgcmVzcGlyYXRpb24gY2FuIGRlcGxldGUgb3h5Z2VuIGNvbmNlbnRyYXRpb25zIGF0IGRlcHRoIHdoZW4gc2Vhc29uYWwgc3RyYXRpZmljYXRpb24gcHJldmVudHMgYWVyYXRpb24gb2YgdGhlIHdhdGVyIGNvbHVtbls1XShodHRwczovL3d3dy5uYXR1cmUuY29tL2FydGljbGVzL3M0MTU1OC0wMjQtMDIwNTgtMyNyZWYtQ1I1KS4gVGhpcyBkZXRyaW1lbnRhbCBjb25kaXRpb24sIHRlcm1lZCBoeXBveGlhIChET%2BKAiTzigIky4oCJbWfigIls4oiSMSksIGRpbWluaXNoZXMgdGhlIGNvbGQtd2F0ZXIgaGFiaXRhdCBvZiBhcXVhdGljIGxpZmUsIGluY2x1ZGluZyBjb21tZXJjaWFsbHkgaW1wb3J0YW50IGZpc2hbNl0oaHR0cHM6Ly93d3cubmF0dXJlLmNvbS9hcnRpY2xlcy9zNDE1NTgtMDI0LTAyMDU4LTMjcmVmLUNSNikgYW5kIGNhbiBjYXVzZSBtYXNzIG1vcnRhbGl0eSBpZiBwZXJzaXN0ZW50WzddKGh0dHBzOi8vd3d3Lm5hdHVyZS5jb20vYXJ0aWNsZXMvczQxNTU4LTAyNC0wMjA1OC0zI3JlZi1DUjcpLiBBbm94aWEgKERP4oCJPOKAiTAuNeKAiW1n4oCJbOKIkjEpIHByb21vdGVzIHRoZSByZWR1Y3RpdmUgZGlzc29sdXRpb24gb2YgbWluZXJhbC1ib3VuZCBjYXJib24gYW5kIHBob3NwaG9ydXNbOF0oaHR0cHM6Ly93d3cubmF0dXJlLmNvbS9hcnRpY2xlcy9zNDE1NTgtMDI0LTAyMDU4LTMjcmVmLUNSOCksIFs5XShodHRwczovL3d3dy5uYXR1cmUuY29tL2FydGljbGVzL3M0MTU1OC0wMjQtMDIwNTgtMyNyZWYtQ1I5KSB3aGljaCBmdWVscyBoYXJtZnVsIGFsZ2FsIGJsb29tc1sxMF0oaHR0cHM6Ly93d3cubmF0dXJlLmNvbS9hcnRpY2xlcy9zNDE1NTgtMDI0LTAyMDU4LTMjcmVmLUNSMTApIGFuZCBsaW1pdHMgbWljcm9iaWFsIG94aWRhdGlvbiBvZiBtZXRoYW5lLCBhIHBvdGVudCBncmVlbmhvdXNlIGdhc1sxMV0oaHR0cHM6Ly93d3cubmF0dXJlLmNvbS9hcnRpY2xlcy9zNDE1NTgtMDI0LTAyMDU4LTMjcmVmLUNSMTEpLiJ9XQ%3D%3D. Climate feedbacks amplified severity, projecting intensified events under RCP scenarios (Nõges et al., 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRpaW5hIiwiZmFtaWx5IjoiTsO1Z2VzIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hc2ltZSIsImZhbWlseSI6IkphbmF0aWFuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlJlZXQiLCJmYW1pbHkiOiJMYXVnYXN0ZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQZWV0ZXIiLCJmYW1pbHkiOiJOw7VnZXMifX1dLCJpZHMiOnsiaXNzbiI6WyIyMzUxLTk4OTQiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMTYvai5nZWNjby4yMDIwLmUwMTM2OSIsIm1hZyI6IjMxMDU0NDYwOTAiLCJvcGVuYWxleCI6IlczMTA1NDQ2MDkwIiwiZXhhIjoiaHR0cHM6Ly93d3cuc2NpZW5jZWRpcmVjdC5jb20vc2NpZW5jZS9hcnRpY2xlL3BpaS9TMjM1MTk4OTQyMDMwOTEwMCJ9LCJ0aXRsZSI6IlBvc3Qtc292aWV0IGNoYW5nZXMgaW4gbml0cm9nZW4gYW5kIHBob3NwaG9ydXMgc3RvaWNoaW9tZXRyeSBpbiB0d28gbGFyZ2Ugbm9uLXN0cmF0aWZpZWQgbGFrZXMgYW5kIHRoZSBpbXBhY3Qgb24gcGh5dG9wbGFua3RvbiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2ouZ2VjY28uMjAyMC5lMDEzNjkiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2ouZ2VjY28uMjAyMC5lMDEzNjkiLCJpc3N1ZWREYXRlIjoiMjAyMC0xMS0yMCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTEiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6Ikdsb2JhbCBFY29sb2d5IGFuZCBDb25zZXJ2YXRpb24iLCJ2b2x1bWUiOjI0LCJpbXBhY3RGYWN0b3IiOiIzLjU5In0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkVsc2V2aWVyIEJWIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo3MSwiY2l0ZWRCeUNvdW50IjoxNH19LCJxdW90ZSI6IlJhbmRvbSBGb3Jlc3QgYW5hbHlzaXMgaW5kaWNhdGVkIGFuIGltcG9ydGFudCByb2xlIG9mIGxpZ2h0IGxpbWl0YXRpb24gaW4gYm90aCBsYWtlcy4gQ29uc3RhbnRseSBsb3dlciBsZXZlbHMgb2YgTiBpbiB0aGUgZGVlcGVyIExha2UgUGVpcHNpIGZhdm91cmVkIE4yLWZpeGluZyBzcGVjaWVzLCB3aGljaCwgYXMgYSBwYXJhZG94LCBiZWNhbWUgUC1saW1pdGVkLiBBcyBjbGltYXRlIHdhcm1pbmcgcmVpbmZvcmNlcyBldXRyb3BoaWNhdGlvbiBwaGVub21lbmEgaW4gbGFrZXMgYnkgaW5jcmVhc2luZyBpbnRlcm5hbCBudXRyaWVudCBsb2FkaW5nIGFuZCBmYXZvdXJpbmcgYmxvb20tZm9ybWluZyBjeWFub2JhY3RlcmlhLCBtb3JlIHN0cmluZ2VudCBtZWFzdXJlcyB3b3VsZCBiZSBuZWVkZWQgdG8gZnVydGhlciBsaW1pdCBudXRyaWVudCBsb2FkcyAoZXNwZWNpYWxseSB0aGF0IG9mIFApIHRvIGxha2VzIHRocm91Z2ggaW1wcm92ZWQgd2FzdGV3YXRlciB0cmVhdG1lbnQgYW5kIGluY3JlYXNlZCBlZmZpY2llbmN5IG9mIGZlcnRpbGlzZXIgYXBwbGljYXRpb24uIExvbmctdGVybSBjaGFuZ2VzIGluIG51dHJpZW50IGxvYWRpbmcgYW5kIHN0b2ljaGlvbWV0cnkgaW4gd2F0ZXIgYm9kaWVzIHJlZmxlY3QgY2hhbmdlcyBpbiBhZ3JpY3VsdHVyYWwgcHJhY3RpY2VzIGFuZCB3YXN0ZXdhdGVyIHRyZWF0bWVudC4ifV0%3D.

Dal Lake in Srinagar, Kashmir, faces perennial algal choking from tourism sewage and siltation, fostering dense scum layers that block 50-70% of navigable channels. Microcystis and Anabaena blooms, peaking post-monsoon, deplete dissolved oxygen to <2 mg/L, triggering mass fish kills of native schizothoracins and economic losses exceeding INR 100 crore yearly in shikara tourism (Nair & Nayak, 2023; Shahnazaryan et al., 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkdheWFuZSIsImZhbWlseSI6IlNoYWhuYXphcnlhbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNYXJ0aW4iLCJmYW1pbHkiOiJTY2h1bHR6ZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLYXJzdGVuIiwiZmFtaWx5IjoiUmlua2UifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQmFyZHVraCIsImZhbWlseSI6IkdhYnJpZWx5YW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIxMTI5LTU3NjciLCIxNzIzLTg2MzMiXSwiaXNibiI6W10sImRvaSI6IjEwLjQwODEvamxpbW5vbC4yMDIyLjIxNjgiLCJvcGVuYWxleCI6Ilc0Mzg4OTc3MDMyIn0sInRpdGxlIjoiTGFrZSBTZXZhbi4gUGFzdCwgcHJlc2VudCwgYW5kIGZ1dHVyZSBzdGF0ZSBvZiBhIHVuaXF1ZSBhbHBpbmUgbGFrZSIsImFic3RyYWN0IjoiTGFrZSBTZXZhbiwgYSBsYXJnZSwgZGVlcCwgYWxwaW5lIGxha2UgaW4gdGhlIExlc3NlciBDYXVjYXN1cyBpcyB0aGUgZm9jdXMgb2YgdGhpcyBTcGVjaWFsIElzc3VlIG9mIHRoZSBKb3VybmFsIG9mIExpbW5vbG9neS4gSXQgd2FzIGFuIG91dHN0YW5kaW5nIGVjb3N5c3RlbSAxMDAgeWVhcnMgYWdvIGNoYXJhY3RlcmlzZWQgYnkgZXhjZWxsZW50IHdhdGVyIHF1YWxpdHksIHJpY2ggYmlvZGl2ZXJzaXR5IHdpdGggYSBoaWdoIGxldmVsIG9mIGVuZGVtaXNtLCB3aWRlLXJhbmdpbmcgYmVkcyBvZiBtYWNyb3BoeXRlcyBhbG9uZyB0aGUgc2hvcmVzIGFuZCBhIHByb2R1Y3RpdmUgYW5kIHN1c3RhaW5hYmxlIGZpc2ggcHJvZHVjdGlvbi4gRHVlIHRvIGl0cyBiZWF1dHksIG5hdHVyYWwgaGlzdG9yeSwgYW5kIGNvbnRyaWJ1dGlvbnMgdG8gc29jaWFsIGFuZCBlY29ub21pYyB3ZWxmYXJlIGl0IGlzIGFsc28gYSBjdWx0dXJhbCBoZXJpdGFnZSBmb3IgdGhlIEFybWVuaWFuIE5hdGlvbiBpbmNsdWRpbmcgaXRzIGxhcmdlIGRpYXNwb3JhLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC40MDgxL2psaW1ub2wuMjAyMi4yMTY4IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5qbGltbm9sLml0L2luZGV4LnBocC9qbGltbm9sL2FydGljbGUvZG93bmxvYWQvMjE2OC8yMDA3IiwiaXNzdWVkRGF0ZSI6IjIwMjMtMTEtMjMiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA5IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJKb3VybmFsIG9mIExpbW5vbG9neSIsInZvbHVtZSI6ODEsImltcGFjdEZhY3RvciI6IjIuMDEifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiUEFHRVByZXNzIChJdGFseSkifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjM1LCJjaXRlZEJ5Q291bnQiOjJ9fSwicXVvdGUiOiJDbGltYXRlIHdhcm1pbmcgY2F1c2VzIGNoYW5nZXMgaW4gdGhlIHRoZXJtYWwgcmVnaW1lIG9mIGxha2VzLCBpLmUuLCB0aGUgdHlwaWNhbCB0ZW1wZXJhdHVyZSBzdHJ1Y3R1cmVzIChXb29sd2F5IGFuZCBNZXItY2hhbnQsIDIwMTkpIGFuZCB0aGUgdGltaW5nIG9mIG1peGluZyBhbmQgc3RyYXRpZmljYXRpb24gKEtyYWVtZXIgZXQgYWwuLCAyMDIxKSwgdGhlIGR1cmF0aW9uIG9mIHN1bW1lciBzdHJhdGlmaWNhdGlvbiAgYW5kIGljZSBjb3ZlciAoUm9nb3JhIGV0IGFsLiwgMjAxODsgU2hhcm1hIGV0IGFsLiwgMjAxOSkgYW5kLCBldmVudHVhbGx5LCBpdCBpcyBhY2NlbGVyYXRpbmcgaW50ZXJuYWwgbnV0cmllbnQgbG9hZGluZyAoTm9ydGggZXQgYWwuLCAyMDE0KSBhbmQgaW1wYWN0aW5nIHRoZSBlbnRpcmUgbGFrZSBlY29zeXN0ZW0gIChTY2h3ZWZlbCBldCBhbC4sIDIwMTk7IFdvb2x3YXkgZXQgYWwuLCAyMDIyKS4gUmVjZW50IHN0dWRpZXMgd29ybGR3aWRlIGRvY3VtZW50ZWQgdW5lcXVpdm9jYWxseSB0aGF0IHRoZSB0eXBpY2FsIHN0cmVzc29ycyBvZiBsYWtlcyBpbnRlcmFjdCwgcG90ZW50aWFsbHkgbGVhZC1pbmcgdG8gZW5mb3JjZWQgbmVnYXRpdmUgY29uc2VxdWVuY2VzLiBUaGVyZSBpcyBjb25zaWRlci1hYmxlIGV2aWRlbmNlIHRoYXQgY2xpbWF0ZSBjaGFuZ2UgaXMgZXhhY2VyYmF0aW5nICBldXRyb3BoaWNhdGlvbiBhbmQgSEFCcyAoUGVhcmwgYW5kIEh1aXNtYW4sIDIwMDg7IFdvb2wtd2F5IGV0IGFsLiwgMjAyMDsgTWVlcmhvZmYgZXQgYWwuLCAyMDIyKSBhbmQgb3RoZXIgZWZmZWN0cyAga25vd24gZnJvbSBldXRyb3BoaWNhdGlvbiBsaWtlIGh5cG94aWEgYW5kIGFub3hpYSBvZiB0aGUgIGh5cG9saW1uaW9uIG9mIHN0cmF0aWZpZWQgbGFrZXMgKEZ1a3VzaGltYSBldCBhbC4sIDIwMTc7ICBSb2dvcmEgZXQgYWwuLCAyMDE4OyBKYW5lIGV0IGFsLiwgMjAyMSkuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NTQuMjUwMjMyNjk2NTMzMiwibGVmdCI6Ny4xNDMzOTMwMzk3MDMzNjksIndpZHRoIjozOC40OTkyNjc1NzgxMjUsImhlaWdodCI6MS4zOTUyNjMzMTQyNDcxMzEzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6MjUuNTIyNzkyODE2MTYyMTEsImxlZnQiOjQ5Ljk5OTE0NTUwNzgxMjUsIndpZHRoIjozOC41MDYyOTQyNTA0ODgyOCwiaGVpZ2h0IjoxMi4xMjk3NzUwNDczMDIyNDZ9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjozOS4zMjI2ODUyNDE2OTkyMiwibGVmdCI6NTIuMzgxMjM3MDMwMDI5Mywid2lkdGgiOjM2LjEyNDQ5NjQ1OTk2MDk0LCJoZWlnaHQiOjEuMzk1MjYzMzE0MjQ3MTMxM319LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjQwLjg1NjAxNDI1MTcwODk4NCwibGVmdCI6NTAuMDAwODIzOTc0NjA5Mzc1LCJ3aWR0aCI6MzguMDg0NDk1NTQ0NDMzNTk0LCJoZWlnaHQiOjEuMzk1MjYzMzE0MjQ3MTMxM319LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjQyLjM4OTMzNTYzMjMyNDIyLCJsZWZ0Ijo1MC4wMDA4MjM5NzQ2MDkzNzUsIndpZHRoIjozOC4wNDg1NTcyODE0OTQxNCwiaGVpZ2h0IjoxLjM5NTI2MzMxNDI0NzEzMTN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo0My45MjI2NjA4Mjc2MzY3MiwibGVmdCI6NTAuMDAwODIzOTc0NjA5Mzc1LCJ3aWR0aCI6MzguNTA1NTEyMjM3NTQ4ODMsImhlaWdodCI6MS4zOTUyNjMzMTQyNDcxMzEzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NDUuNDU1OTg2MDIyOTQ5MjIsImxlZnQiOjUwLjAwMDgyMzk3NDYwOTM3NSwid2lkdGgiOjM4LjA4MDc4MDAyOTI5Njg3NSwiaGVpZ2h0IjoxLjM5NTI2MzMxNDI0NzEzMTN9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo0Ni45OTA1NjYyNTM2NjIxMSwibGVmdCI6NTAuMDAwODIzOTc0NjA5Mzc1LCJ3aWR0aCI6MzguNDk5OTQ2NTk0MjM4MjgsImhlaWdodCI6MS4zOTUyNjMzMTQyNDcxMzEzfX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NDguNTIyNjI4Nzg0MTc5NjksImxlZnQiOjUwLjAwMDgyMzk3NDYwOTM3NSwid2lkdGgiOjM4LjUwMzExMjc5Mjk2ODc1LCJoZWlnaHQiOjEuMzk1MjYzMzE0MjQ3MTMxM319LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjUwLjA1NzIxMjgyOTU4OTg0NCwibGVmdCI6NTAuMDAwODIzOTc0NjA5Mzc1LCJ3aWR0aCI6MzguNDk4OTM5NTE0MTYwMTU2LCJoZWlnaHQiOjEuMzk1MjYzMzE0MjQ3MTMxM319LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjUxLjU5MDUzODAyNDkwMjM0NCwibGVmdCI6NTAuMDAwODI3Nzg5MzA2NjQsIndpZHRoIjozOC41MDEwNzk1NTkzMjYxNywiaGVpZ2h0IjoxLjM5NTI2MzMxNDI0NzEzMTN9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIxIn19LHsib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hdnlhIFZpa3JhbWFuIiwiZmFtaWx5IjoiTmFpciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQcmF0ZWVwIEt1bWFyIiwiZmFtaWx5IjoiTmF5YWsifX1dLCJpZHMiOnsiaXNzbiI6WyIyMjk2LTc3NDUiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzODkvZm1hcnMuMjAyMy4xMDg3Mjk2Iiwib3BlbmFsZXgiOiJXNDM4MjcyMzEzMyJ9LCJ0aXRsZSI6IlVuY292ZXJpbmcgd2F0ZXIgcXVhbGl0eSBhbmQgZXZhbHVhdGluZyB2dWxuZXJhYmlsaXRpZXMgb2Ygc21hbGwtc2NhbGUgZmlzaGVyaWVzIGluIENoaWxpa2EgTGFnb29uLCBJbmRpYSIsImFic3RyYWN0IjoiU21hbGwtc2NhbGUgZmlzaGVyaWVzIChTU0ZzKSBwbGF5IGEgc3Ryb25nIHJvbGUgaW4gc3VzdGFpbmluZyBtaWxsaW9ucyBvZiBsaXZlbGlob29kcywgZm9vZCBzZWN1cml0eSwgbnV0cml0aW9uLCBhbmQgaW5jb21lIGdsb2JhbGx5IGJ1dCB0aGUgZmlzaGVycyBlbmdhZ2VkIGluIHRoaXMgc2VjdG9yIHNpbXVsdGFuZW91c2x5IGV4cGVyaWVuY2UgaGlnaCBsZXZlbHMgb2YgdnVsbmVyYWJpbGl0eSBhbmQgcHJvY2Vzc2VzIG9mIG1hcmdpbmFsaXNhdGlvbi4gU2V2ZXJhbCBmYWN0b3JzIGFyZSBhdHRyaWJ1dGVkIHRvIHRoZSBtdWx0aWRpbWVuc2lvbmFsIHZ1bG5lcmFiaWxpdGllcyB0aGUgc21hbGwtc2NhbGUgZmlzaGVycyBleHBlcmllbmNlLCBzcGFubmluZyBib3RoIG5hdHVyYWwgKGUuZy4sIG5hdHVyYWwgZGlzYXN0ZXJzLCBlY29zeXN0ZW0gY2hhbmdlKSBhbmQgYW50aHJvcG9nZW5pYyAoZS5nLiwgcG9saWN5IGNoYW5nZSwgaHlkcm9sb2dpY2FsIGludGVydmVudGlvbnMsIGFxdWFjdWx0dXJlKSBwcmVzc3VyZXMuIFdoaWxlIHRoZXJlIGlzIG11Y2ggbGl0ZXJhdHVyZSBvbiB2YXJpb3VzIG5hdHVyYWwgYW5kIGh1bWFuIGRyaXZlcnMgb2YgdnVsbmVyYWJpbGl0eSBpbiBzbWFsbC1zY2FsZSBmaXNoZXJ5IGNvbW11bml0aWVzLCBhbiBhYnNlbmNlIG9mIHJlc2VhcmNoIGNvbm5lY3RpbmcgdnVsbmVyYWJpbGl0eSB3aXRoIHdhdGVyIHF1YWxpdHkgaXMgZXZpZGVudC4gRmlzaGVyIGNvbW11bml0aWVzIG9mdGVuIHRhbGsgYWJvdXQgZmlzaCBpbiByZWxhdGlvbiB0byB0aGUgaGVhbHRoIG9mIHRoZWlyIGFxdWF0aWMgaGFiaXRhdHMgd2hlcmVpbiB3YXRlciBxdWFsaXR5IGlzIHNlZW4gYXMgYSBrZXkgcGFyYW1ldGVyLiBUaGUgbGluayBiZXR3ZWVuIGhlYWx0aHkgZmlzaCBhbmQgZ29vZCBxdWFsaXR5IHdhdGVyIGhhcyBzaWduaWZpY2FudCBpbXBsaWNhdGlvbnMgZm9yIHN0cm9uZyBhbmQgdmlhYmxlIGZpc2hpbmcgY29tbXVuaXRpZXMuIFRoaXMgcGFwZXIgZXhhbWluZXMgdGhlc2UgbGlua3MgZnVydGhlciBieSBmb2N1c2luZyBvbiB0aGUgbmF0dXJlIG9mIHZ1bG5lcmFiaWxpdGllcyBjYXVzZWQgYnkgd2F0ZXIgcXVhbGl0eSBjaGFuZ2VzIGluIHRoZSBzbWFsbC1zY2FsZSBmaXNoZXJ5IHN5c3RlbSBvZiBDaGlsaWthIExhZ29vbiBpbiBJbmRpYS4gV2UgdW5kZXJ0YWtlIGRldGFpbGVkIGFuYWx5c2lzIG9mIHRoZSBpbnZhc2l2ZSBzaHJpbXAgYXF1YWN1bHR1cmUgYWN0aXZpdGllcyBhbmQgaHlkcm9sb2dpY2FsIGludGVydmVudGlvbnMgZm9yIG9wZW5pbmcgb2YgYSBsYWdvb24gaW5sZXQgd2l0aCB0aGUgQmF5IG9mIEJlbmdhbCBhcyB0d28gZG9taW5hbnQgZHJpdmVycyBhZHZlcnNlbHkgaW1wYWN0aW5nIHdhdGVyIHF1YWxpdHkgYW5kIGluY3JlYXNpbmcgdnVsbmVyYWJpbGl0aWVzIG9mIHRoZSBlbnRpcmUgc21hbGwtc2NhbGUgZmlzaGVyaWVzIHNvY2lhbC1lY29sb2dpY2FsIHN5c3RlbS4gT3VyIGFuYWx5c2lzIHN1Z2dlc3RzIHRoYXQgdGhlcmUgYXJlIHN0cm9uZyBpbnRlcmNvbm5lY3Rpb25zIGJldHdlZW4gY2hhbmdlcyBpbiB3YXRlciBxdWFsaXR5IGFuZCB0aGUgbGV2ZWxzIG9mIHZ1bG5lcmFiaWxpdGllcyBpbiB0aGUgU1NGcyBvZiBDaGlsaWthIExhZ29vbi4gUG9sbHV0YW50cyBzdWNoIGFzIHBlc3RpY2lkZXMsIGFuZCBvcmdhbmljIGNvbXBvdW5kcyBhY2N1bXVsYXRlIGluIGZpc2ggdGlzc3VlcyBhbmQgYWZmZWN0IHRoZWlyIGdyb3d0aCwgcmVwcm9kdWN0aW9uLCBhbmQgb3ZlcmFsbCBoZWFsdGguIFRoaXMgbGVkIHRvIGRlY2xpbmVzIGluIGZpc2ggcG9wdWxhdGlvbnMsIG1ha2luZyBpdCBtb3JlIGRpZmZpY3VsdCBmb3IgZmlzaGVycyB0byBtYWtlIGEgbGl2aW5nLiBJbiBhZGRpdGlvbiB0byBkaXJlY3QgaW1wYWN0cyBvbiBmaXNoIHBvcHVsYXRpb25zLCBwb29yIHdhdGVyIHF1YWxpdHkgYWxzbyBoYXMgaW5kaXJlY3QgZWZmZWN0cyBvbiB0aGUgc29jaWFsIGFuZCBlY29ub21pYyB2dWxuZXJhYmlsaXR5IG9mIFNTRnMuIEZvciBleGFtcGxlLCBjb250YW1pbmF0aW9uIG9mIHdhdGVyIHNvdXJjZXMgbGVkIHRvIHRoZSByZWR1Y2VkIG51bWJlciBvZiBmaXNoIHNwZWNpZXMgcmVkdWNpbmcgdGhlIGFtb3VudCBvZiB0aW1lIGZpc2hlcnMgY2FuIHNwZW5kIG9uIGZpc2hpbmcgYWN0aXZpdGllcy4gVGhpcyBhbHNvIGFmZmVjdGVkIHRoZSBtYXJrZXRhYmlsaXR5IG9mIGZpc2ggcHJvZHVjdHMsIHJlZHVjZWQgaW5jb21lIGFuZCBpbmNyZWFzZWQgcG92ZXJ0eS4gVG8gZnVsbHkgdW5kZXJzdGFuZCB0aGUgaW50ZXJjb25uZWN0aW9ucyBiZXR3ZWVuIHdhdGVyIHF1YWxpdHkgYW5kIHZ1bG5lcmFiaWxpdHkgaW4gU1NGcyBpbiBDaGlsaWthIExhZ29vbiwgaXQgaXMgaW1wb3J0YW50IHRvIGNvbnNpZGVyIGJvdGggZW52aXJvbm1lbnRhbCBhbmQgc29jaWFsIGZhY3RvcnMsIGFzIHdlbGwgYXMgdGhlIGNvbXBsZXggZmVlZGJhY2sgbG9vcHMgYmV0d2VlbiB0aGVzZSBmYWN0b3JzLiBUaGUgc3R1ZHkgaGVscHMgaW4gYnJpZGdpbmcgYSBjcnVjaWFsIGdhcCBpbiBvdXIgdW5kZXJzdGFuZGluZyBvZiB0aGUgcm9sZSBvZiB3YXRlciBxdWFsaXR5IGluIHZ1bG5lcmFiaWxpdHkgYW5hbHlzaXMgd2l0aGluIHJlc291cmNlIGRlcGVuZGVudCBjb21tdW5pdGllcy4gV2UgY29uY2x1ZGUgd2l0aCBrZXkgaW5zaWdodHMgb24gcG9zc2libGUgY29waW5nIHJlc3BvbnNlcyBhbmQgYWRhcHRpdmUgY2FwYWNpdHkgbmVjZXNzYXJ5IGZvciB0aGUgc21hbGwtc2NhbGUgZmlzaGVyaWVzIGNvbW11bml0aWVzIHRvIHRyYW5zaXRpb24gdG93YXJkIHZpYWJpbGl0eS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMzM4OS9mbWFycy4yMDIzLjEwODcyOTYiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vd3d3LmZyb250aWVyc2luLm9yZy9hcnRpY2xlcy8xMC4zMzg5L2ZtYXJzLjIwMjMuMTA4NzI5Ni9wZGYiLCJpc3N1ZWREYXRlIjoiMjAyMy0wNi0yOSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZyb250aWVycyBpbiBNYXJpbmUgU2NpZW5jZSIsInZvbHVtZSI6MTAsImltcGFjdEZhY3RvciI6IjQuMjgifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRnJvbnRpZXJzIE1lZGlhIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo1MSwiY2l0ZWRCeUNvdW50Ijo1fX0sInF1b3RlIjoiV2F0ZXIgcXVhbGl0eSBvZiBDaGlsaWthIExhZ29vbiBoYXMgZGVncmFkZWQgZHVlIHRvIGFudGhyb3BvZ2VuaWMgaW50ZXJ2ZW50aW9ucyBhbmQgbmF0dXJhbCBmb3JjZXMgb2YgY2hhbmdlLiBWYXJpb3VzIGFjdGl2aXRpZXMgaW4gdGhlIENoaWxpa2EgTGFnb29uLCBzdWNoIGFzIGluZHVzdHJpYWwgd2FzdGV3YXRlciBkaXNwb3NhbCwgc2V3YWdlIGR1bXBpbmcsIGFxdWFjdWx0dXJlLCBoeWRyb2xvZ2ljYWwgaW50ZXJ2ZW50aW9ucywgYW5kIGN5Y2xvbmVzLCBhcmUgcHJvZHVjaW5nIHNhbGluaXR5IGNoYW5nZXMsIHNlZGltZW50IGRlcG9zaXRpb24sIG51dHJpZW50IGVucmljaG1lbnQsIGV1dHJvcGhpY2F0aW9uLCBhbmQgZGVhZCB6b25lcywgYXMgc3RhdGVkLiBGaXNoIHN1cnZpdmFsIGlzIGF0IHJpc2sgYXMgYSByZXN1bHQgb2YgdGhlIHNpZ25pZmljYW50IHJlZHVjdGlvbiBpbiBlbnZpcm9ubWVudGFsIGNpcmN1bXN0YW5jZXMsIHdoaWNoIGRpbWluaXNoZXMgZmlzaGVybWVu4oCZcyBpbmNvbWUgYW5kIHB1c2hlcyB0aGVtIG91dCBvZiBzb2NpZXR5LiBUaGVzZSBjYXVzZXMgYWZmZWN0aW5nIHRoZSB3YXRlciBxdWFsaXR5IG9mIHRoZSBMYWdvb24gZGVwaWN0IG1hbnkgZmFjZXMgb2YgdnVsbmVyYWJpbGl0eSBpbiBTU0YgY29tbXVuaXRpZXMuIEFzIHRoZSBDaGlsaWthIExhZ29vbiBlbnZpcm9ubWVudCBpcyBleHBlY3RlZCB0byB1bmRlcmdvIGluY3JlYXNpbmdseSBkcmFzdGljIGNoYW5nZXMgKHN1Y2ggYXMgY2F0Y2ggY29tcG9zaXRpb24sIGNhdGNoIGNhcGFjaXR5LCBiaW9kaXZlcnNpdHkgdmFyaWF0aW9ucywgd2F0ZXIgcXVhbGl0eSwgYW5kIGZpc2hlcnkgcmV2ZW51ZXMpLCBpbnRlcm5hdGlvbmFsIHBvbGljeSBoYXMgYmVjb21lIG1vcmUgaW50ZXJlc3RlZCBpbiBhZGFwdGF0aW9uIG1lYXN1cmVzLiBHaXZlbiBob3cgZGl2ZXJzZSBTU0YgZWNvc3lzdGVtcyBhcmUgZXhwZWN0ZWQgdG8gYmUgYmV0d2VlbiBjb250aW5lbnRzLCB0aGVyZSBpcyBubyDigJxvbmUgc2l6ZSBmaXRzIGFsbOKAnSBhcHByb2FjaC4gVGhlIHBhcGVyIG91dGxpbmVkIHNldmVyYWwgY29waW5nIGFuZCBhZGFwdGF0aW9uIG1lY2hhbmlzbXMgdG8gaW1wcm92ZSB3YXRlciBxdWFsaXR5IGFuZCBjb250aW51ZSBmaXNoaW5nIGluIGEgc3VzdGFpbmFibGUgd2F5LiBBZGFwdGF0aW9uIHN0cmF0ZWdpZXMgY2FuIG9jY2FzaW9uYWxseSBoYXZlIHVuZm9yZXNlZW4gY29uc2VxdWVuY2VzIGZvciBmaXNoZXJpZXMgYW5kIGZpc2hpbmcgY29tbXVuaXRpZXMuIEFueSB1bmV4cGVjdGVkIGNvbnNlcXVlbmNlcyBvZiBhZGFwdGF0aW9uIGNhbiBiZSBtaXRpZ2F0ZWQgdGhyb3VnaCBsb25nLXRlcm0gcGxhbm5pbmcgYW5kIGlkZW50aWZpY2F0aW9uIG9mIGZ1dHVyZSBvcHRpb25zIHRocm91Z2ggc2NlbmFyaW8gYW5hbHlzaXMuIFRoZSBmaW5kaW5ncyBvZiB0aGlzIHBhcGVyIGNhbiBiZSB1dGlsaXplZCB0byBkZXZlbG9wIHBvbGljaWVzIHRvIHByb3RlY3Qgd2F0ZXIgcXVhbGl0eSBhbmQgU1NGcyBpbiBhIHZhcmlldHkgb2YgY29hc3RhbCBsYWdvb24gc2V0dGluZ3MuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjIyLjI4NTAzNjA4NzAzNjEzMywibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ5MzA1MzQzNjI3OTMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjIzLjc3MzUxOTUxNTk5MTIxLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDgzMzY0MTA1MjI0NjEsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjI1LjI1NTU4NDcxNjc5Njg3NSwibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ4OTU1OTE3MzU4Mzk4NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjYuNzQ0MDY4MTQ1NzUxOTUzLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNTA2ODA1NDE5OTIxODc1LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyOC4yMjYxMzMzNDY1NTc2MTcsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40OTA0NjMyNTY4MzU5NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjkuNzE0NjE2Nzc1NTEyNjk1LCJsZWZ0Ijo4LjMzMTU5MjU1OTgxNDQ1Mywid2lkdGgiOjQwLjQ5NDA5ODY2MzMzMDA4LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozMS4xOTY2ODM4ODM2NjY5OTIsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40OTQzOTIzOTUwMTk1MywiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MzIuNjg1MTY1NDA1MjczNDQsImxlZnQiOjguMzMxNTk2Mzc0NTExNzE5LCJ3aWR0aCI6NDAuNDg3NDQ1ODMxMjk4ODMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM0LjE2NzIzMjUxMzQyNzczNCwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40OTc3NDU1MTM5MTYwMTYsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM1LjY1NTcxNTk0MjM4MjgxLCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ5ODYzODE1MzA3NjE3LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozNy4xMzc3ODMwNTA1MzcxMSwibGVmdCI6MTAuNjg1MTg2Mzg2MTA4Mzk4LCJ3aWR0aCI6MzguMTQxMTA1NjUxODU1NDcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM4LjYyNjI2NjQ3OTQ5MjE5LCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4NzE1MjA5OTYwOTM3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDAuMTA4MzMzNTg3NjQ2NDg0LCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4OTg1NjcxOTk3MDcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQxLjU5NjgxMzIwMTkwNDMsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDg4NjY2NTM0NDIzODMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQzLjA3ODg4MDMxMDA1ODU5NCwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40ODE4NjExMTQ1MDE5NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDQuNTY3MzYzNzM5MDEzNjcsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDgzMDU4OTI5NDQzMzYsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQ2LjA0OTQzMDg0NzE2Nzk3LCJsZWZ0Ijo4LjMzMzEwODkwMTk3NzUzOSwid2lkdGgiOjQwLjUwMDc0NzY4MDY2NDA2LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo0Ny41Mzc5MTQyNzYxMjMwNSwibGVmdCI6OC4zMzMxMDg5MDE5Nzc1MzksIndpZHRoIjo0MC40OTU2MDU0Njg3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDkuMDE5OTgxMzg0Mjc3MzQ0LCJsZWZ0Ijo4LjMzMzEwNjA0MDk1NDU5LCJ3aWR0aCI6NDAuNTA2ODE2ODY0MDEzNjcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUwLjUwODQ2NDgxMzIzMjQyLCJsZWZ0Ijo4LjMzMzEwNjA0MDk1NDU5LCJ3aWR0aCI6NDAuNDg4NTA2MzE3MTM4NjcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUxLjk5MDUyODEwNjY4OTQ1LCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDk3NzI2NDQwNDI5NjksImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUzLjQ3OTAxMTUzNTY0NDUzLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6MzMuMTU5NDQ2NzE2MzA4NTk0LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo1NC45NjEwNzg2NDM3OTg4MywibGVmdCI6MTAuNjgzNjcwMDQzOTQ1MzEyLCJ3aWR0aCI6MzguMTM2NTk2Njc5Njg3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NTYuNDQ5NTYyMDcyNzUzOTA2LCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDg3MTQ0NDcwMjE0ODQ0LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEzIn19XQ%3D%3D. Eutrophication, with total phosphorus >100 μg/L, homogenizes biodiversity, favoring invasives over endemic macrophytes, while dermal toxins deter 5 million annual visitors, slashing occupancy by half (Paerl et al., 2020; Shahbaz et al., 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhhbnMgVy4iLCJmYW1pbHkiOiJQYWVybCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLLkUuIiwiZmFtaWx5IjoiSGF2ZW5zIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik4uUy4iLCJmYW1pbHkiOiJIYWxsIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlQuRy4iLCJmYW1pbHkiOiJPdHRlbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNLiIsImZhbWlseSI6IlpodSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJIYWlodWkiLCJmYW1pbHkiOiJYdSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJHLiIsImZhbWlseSI6IlpodSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJCLiIsImZhbWlseSI6IlFpbiJ9fV0sImlkcyI6eyJpc3NuIjpbXSwiaXNibiI6W10sImRvaSI6IjEwLjE3NjE1LzRua2Qtems5MyIsIm9wZW5hbGV4IjoiVzQzMDcwNjI0MjkifSwidGl0bGUiOiJNaXRpZ2F0aW5nIGEgZ2xvYmFsIGV4cGFuc2lvbiBvZiB0b3hpYyBjeWFub2JhY3RlcmlhbCBibG9vbXM6IENvbmZvdW5kaW5nIGVmZmVjdHMgYW5kIGNoYWxsZW5nZXMgcG9zZWQgYnkgY2xpbWF0ZSBjaGFuZ2UiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTc2MTUvNG5rZC16azkzIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2Nkci5saWIudW5jLmVkdS9kb3dubG9hZHMvMDU3NDIyODM0IiwiaXNzdWVkRGF0ZSI6IjIwMjAtMDEtMDEiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTAxIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJDYXJvbGluYSBEaWdpdGFsIFJlcG9zaXRvcnkgKFVuaXZlcnNpdHkgb2YgTm9ydGggQ2Fyb2xpbmEgYXQgQ2hhcGVsIEhpbGwpIn0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlVuaXZlcnNpdHkgb2YgTm9ydGggQ2Fyb2xpbmEgYXQgQ2hhcGVsIEhpbGwifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjAsImNpdGVkQnlDb3VudCI6MH19LCJxdW90ZSI6IlRoYXQgcmVsYXRpb25zaGlwIGhhcyBzZXJ2ZWQgYXMgdGhlIGZvdW5kYXRpb24gZm9yIHNldHRpbmcgIG51dHJpZW50IGxvYWRpbmcgdGFyZ2V0cyBpbiBsYWtlcyBhcm91bmQgdGhlIHdvcmxkLCBhbmQgY2FyZWZ1bGx5ICBkZXNpZ25lZCBleHBlcmltZW50cyBtYXkgZWx1Y2lkYXRlIHRvIHdoYXQgZXh0ZW50IHRlbXBlcmF0dXJlICBpbmNyZWFzZSBhZmZlY3RzIHRoZSB0aHJlc2hvbGQgYXQgd2hpY2ggdGhlIGZyZXF1ZW5jeSBvZiBvY2N1ciAgcmVuY2Ugb2YgQ3lhbm9IQUJzIHF1aWNrbHkgcmlzZXMgd2l0aCBpbmNyZWFzaW5nIE4gYW5kIFAuIENvbmNsdXNpb25zIFJlY2VudCBzdHVkaWVzIGluZGljYXRlIHRoYXQgdGhlIHN5bmVyZ2lzdGljIGNvbWJpbmF0aW9uIG9mIGFudGhyb3BvZ2VuaWMgbnV0cmllbnQgbG9hZGluZywgcmlzaW5nIHRlbXBlcmF0dXJlcywgZW5oYW5jZWQgIHZlcnRpY2FsIHN0cmF0aWZpY2F0aW9uLCBpbmNyZWFzZWQgcmVzaWRlbmNlIHRpbWUgYW5kIG1vcmUgIGV4dHJlbWUgY2xpbWF0aWMgY29uZGl0aW9ucyBvdmVyYWxsIHdpbGwgZmF2b3VyIGN5YW5vYmFjdGVyaWFsICBkb21pbmFuY2UgaW4gYSB3aWRlIHJhbmdlIG9mIGFxdWF0aWMgZWNvc3lzdGVtcy4gVGhlIGV4cGFuICBzaW9uIG9mIEN5YW5vSEFCcyBoYXMgc2VyaW91cyBjb25zZXF1ZW5jZXMgZm9yIGh1bWFuIHdhdGVyICBzdXBwbGllcywgZmlzaGVyaWVzIGFuZCByZWNyZWF0aW9uYWwgcmVzb3VyY2VzLiBFY29zeXN0ZW0gc2NhbGUgIHBoeXNpY2FsIGNoZW1pY2FsIGJpb3RpYyBtaXRpZ2F0aW9uIHN0cmF0ZWdpZXMgd2lsbCBoYXZlIHRvICBpbmNvcnBvcmF0ZSBudXRyaWVudCAobW9zdCBvZnRlbiBib3RoIE4gYW5kIFApIGlucHV0IHJlZHVjdGlvbnMuICBUaGUgbWFnbml0dWRlcyBvZiB0aGVzZSByZWR1Y3Rpb25zIHdpbGwgbmVlZCB0byBiZSBzeXN0ZW0gIHNwZWNpZmljIGFuZCB3aWxsIGxpa2VseSBuZWVkIHRvIGJlIGFkanVzdGVkIHdpdGggY2hhbmdpbmcgY2xpICBtYXRpYyBjb25kaXRpb25zLiBTcGVjaWZpY2FsbHksIHdhdGVyIHF1YWxpdHkgbWFuYWdlcnMgd2lsbCBoYXZlICB0byBhY2NvbW1vZGF0ZSB3YXJtaW5nIGFuZCBncmVhdGVyIGh5ZHJvbG9naWNhbCB2YXJpYWJpbGl0eSAgYW5kIGV4dHJlbWVuZXNzIHJlc3VsdGluZyBmcm9tIGNsaW1hdGUgY2hhbmdlIGluIGZvcm11bGF0aW5nICB0aGVpciBzdHJhdGVnaWVzLiBBIGtleSBsb25nIHRlcm0gY29udHJvbCB3ZSBjYW4gZXhlcnQgdG8gcmVkdWNlICB0aGUgcmF0ZSBhbmQgZXh0ZW50IG9mIGdsb2JhbCB3YXJtaW5nIGlzIGN1cmJpbmcgZ3JlZW5ob3VzZSBnYXMgIGVtaXNzaW9ucy4gV2l0aG91dCB0aGlzIGVzc2VudGlhbCBzdGVwLCBmdXR1cmUgd2FybWluZyB0cmVuZHMsICBoeWRyb2xvZ2ljYWwgZXh0cmVtZXMgKHJlY29yZCByYWluZmFsbHMgYW5kIGRyb3VnaHRzKSBhbmQgdGhlaXIgIGVmZmVjdHMgb24gYXF1YXRpYyBlY29zeXN0ZW1zIHdpbGwgY29udGludWUgdG8gcHJvbW90ZSBvcHBvciAgdHVuaXN0aWMgQ3lhbm9IQUJzLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjoxNC4xNDkyMzM4MTgwNTQyLCJsZWZ0IjoxMy4xMjg1OTUzNTIxNzI4NTIsIndpZHRoIjozNy4wODIxNzIzOTM3OTg4MywiaGVpZ2h0IjoxLjUxNjYwMjI3Nzc1NTczNzN9fSx7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6MTUuNDUxNTA4NTIyMDMzNjkxLCJsZWZ0IjoxMy4xMjg1NDQ4MDc0MzQwODIsIndpZHRoIjozNy4wODQ4MzEyMzc3OTI5NywiaGVpZ2h0IjoxLjUxNjYwMzIzMTQzMDA1Mzd9fSx7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6MTYuNzUzNzg0MTc5Njg3NSwibGVmdCI6MTMuMTI4NTQyOTAwMDg1NDUsIndpZHRoIjozNy4wODMyMDk5OTE0NTUwOCwiaGVpZ2h0IjoxLjUxNjYwNDE4NTEwNDM3MDF9fSx7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6MTguMDU2MDUzMTYxNjIxMDk0LCJsZWZ0IjoxMy4xMjg1NDQ4MDc0MzQwODIsIndpZHRoIjozNi42MTUwNzQxNTc3MTQ4NDQsImhlaWdodCI6MS41MTY2MDQxODUxMDQzNzAxfX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjE5LjQ2ODM3ODA2NzAxNjYsImxlZnQiOjEzLjEyODU0NDgwNzQzNDA4Miwid2lkdGgiOjM0LjI5MDUzMTE1ODQ0NzI2NiwiaGVpZ2h0IjoxLjI0NTAxMTIxMDQ0MTU4OTR9fSx7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6MjIuMjI2MTQyODgzMzAwNzgsImxlZnQiOjEzLjEyODU1MDUyOTQ3OTk4LCJ3aWR0aCI6Ny42NDk4NzM3MzM1MjA1MDgsImhlaWdodCI6MS4xMzU0ODMxNDU3MTM4MDYyfX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjIzLjczNDg0OTkyOTgwOTU3LCJsZWZ0IjoxMy4xMjg1MzUyNzA2OTA5MTgsIndpZHRoIjozNy4wODkyODY4MDQxOTkyMiwiaGVpZ2h0IjoyNi4wODg2NDAyMTMwMTI2OTV9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIxMSJ9fSx7Im9yaWdpbiI6NCwid29yayI6eyJ0eXBlIjo4LCJzdWJ0eXBlIjp7Im9uZW9mS2luZCI6ImFydGljbGVUeXBlIiwiYXJ0aWNsZVR5cGUiOjF9LCJjb250cmlidXRvcnMiOlt7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNZCIsImZhbWlseSI6IlNoYWhiYXoifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQWhiYXIiLCJmYW1pbHkiOiJBbGFtIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1vaGFtbWFkIE1hc3Jvb3IiLCJmYW1pbHkiOiJaYWZhciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNb2hhbW1lZCBBYXNpZiIsImZhbWlseSI6IlN1bGFpbWFuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFudXBtYSIsImZhbWlseSI6Ikt1bWFyaSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJHLiBLLiIsImZhbWlseSI6IlNoYXJtYSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTaGFobGEiLCJmYW1pbHkiOiJZYXNtaW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIxMjE1LTE2MTAiLCIyMDYxLTk1ODgiXSwiaXNibiI6W10sImRvaSI6IjEwLjI0Nzgvb3JodS0yMDIzLTAwMTYiLCJvcGVuYWxleCI6Ilc0Mzg5NTEwMDM3In0sInRpdGxlIjoiQ29tcGFyaXNvbiBvZiBhdmlhbiBkaXZlcnNpdHkgYmV0d2VlbiBtYW5hZ2VkIGFuZCB1bm1hbmFnZWQgd2V0bGFuZHMgaW4gUGF0bmEsIEJpaGFyLCBJbmRpYSIsImFic3RyYWN0IjoiQWJzdHJhY3QgV2UgY29tcGFyZWQgdGhlIGJpcmQgZGl2ZXJzaXR5IG9mIGEgbWFuYWdlZCAoUmFqZGhhbmkgSmFsYXNoYXkpIGFuZCBhbiB1bm1hbmFnZWQgd2V0bGFuZCAoTXVzdGFmYXB1cikgaW4gUGF0bmEsIEJpaGFyLCBJbmRpYS4gV2UgcmVjb3JkZWQgYmlyZCBzcGVjaWVzIGJ5IHVzaW5nIHBvaW50IGNvdW50cyBhbmQgZml4ZWQtcm91dGUgbW9uaXRvcmluZy4gQXQgUmFqZGhhbmkgSmFsYXNoYXksIGEgdG90YWwgb2YgNzMgc3BlY2llcyB3ZXJlIHJlY29yZGVkLCBvZiB3aGljaCAxNSBzcGVjaWVzIHdlcmUgd2ludGVyIHZpc2l0b3JzLiBUd28gc3BlY2llcyB3ZXJlIGluIHRoZSBOZWFyLXRocmVhdGVuZWQgY2F0ZWdvcnk6IEZlcnJ1Z2lub3VzIER1Y2sgKEF5dGh5YSBueXJvY2EpIGFuZCBBbGV4YW5kcmluZSBQYXJha2VldCAoUHNpdHRhY3VsYSBldXBhdHJpYSkgLCB3aGlsZSB0aGUgb3RoZXJzIHdlcmUgb2YgbGVhc3QgY29uY2Vybi4gQXQgTXVzdGFmYXB1ciB3ZXRsYW5kLCA2NyBzcGVjaWVzIHdlcmUgcmVjb3JkZWQgd2l0aCAxMSBzcGVjaWVzIGFzIHdpbnRlciB2aXNpdG9ycy4gU2ltcHNvbuKAmXMgc3BlY2llcyBkaXZlcnNpdHkgaW5kZXggd2FzIDAuNzggYXQgUmFqZGhhbmkgSmFsYXNoYXksIGFuZCAwLjgxIGF0IE11c3RhZmFwdXIgd2V0bGFuZC4gVGhlIG51bWJlciBvZiBzcGVjaWVzIG9mIHdhZGVycyB3YXMgMTcgYXQgTXVzdGFmYXB1ciB3ZXRsYW5kIGFuZCAxMSBhdCBSYWpkaGFuaSBKYWxhc2hheS4gVGhlIHRvdGFsIG51bWJlciBvZiBMZXNzZXIgV2hpc3RsaW5nLWR1Y2sgKERlbmRyb2N5Z25hIGphdmFuaWNhKSAsIGEgcmVzaWRlbnQgYmlyZCB3YXMgYWxtb3N0IGRvdWJsZSAoNTI2KSBhdCBNdXN0YWZhcHVyIHdldGxhbmQgdGhhbiB0aGF0IGF0IFJhamRoYW5pIEphbGFzaGF5ICgyMzQpLiBUaGUgbml0cmF0ZSBsZXZlbCB3YXMgdHdpY2UgaGlnaGVyIGFuZCB0aGUgcGhvc3BoYXRlIGxldmVsIHdhcyBmb3VyIHRpbWVzIGhpZ2hlciBhdCBNdXN0YWZhcHVyIHdldGxhbmQgdGhhbiB0aG9zZSBhdCBSYWpkaGFuaSBKYWxhc2hheS4gVGhlIGRpZmZlcmVuY2UgaW4gc3BlY2llcyBudW1iZXIgYW5kIGNvbXBvc2l0aW9uIGJldHdlZW4gdGhlIHR3byB3ZXRsYW5kcyBtYXkgYmUgYXR0cmlidXRlZCB0byB0aGUgZXV0cm9waGljYXRpb24gcmVzdWx0aW5nIGZyb20gaGlnaCBuaXRyYXRlIGFuZCBwaG9zcGhhdGUgbGV2ZWxzIGF0IE11c3RhZmFwdXIgd2V0bGFuZC4gVW5mb3J0dW5hdGVseSwgdGhlIE11c3RhZmFwdXIgd2V0bGFuZCBpcyB1bmRlciB0aHJlYXQgZHVlIHRvIGh1bWFuIGluZmx1ZW5jZXMuIFRoZSBwcm90ZWN0aW9uIGFuZCBzdXN0YWluYWJsZSBtYW5hZ2VtZW50IG9mIG5hdHVyYWwgd2V0bGFuZHMgaXMgcmVxdWlyZWQgZm9yIHNhdmluZyB0aGUgYmlvZGl2ZXJzaXR5IG9mIHRoZSBhcmVhLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4yNDc4L29yaHUtMjAyMy0wMDE2IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3NjaWVuZG8uY29tL3BkZi8xMC4yNDc4L29yaHUtMjAyMy0wMDE2IiwiaXNzdWVkRGF0ZSI6IjIwMjMtMTItMDEiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA5IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJPcm5pcyBIdW5nYXJpY2EiLCJpc3N1ZSI6Miwidm9sdW1lIjozMSwiaW1wYWN0RmFjdG9yIjoiMC45MiJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjEzLCJsYXN0UGFnZSI6Mjh9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiSHVuZ2FyaWFuIE9ybml0aG9sb2dpY2FsIGFuZCBOYXR1cmUgQ29uc2VydmF0aW9uIFNvY2lldHkifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjI3LCJjaXRlZEJ5Q291bnQiOjF9fSwicXVvdGUiOiJIb3dldmVyLCBoaWdoIGxldmVscyBvZiBldXRyb3BoaWNhdGlvbiBtYXkgYWxzbyB0cmlnZ2VyIHRoZSBwcm9saWZlcmF0aW9uIG9mIGdyZWVuIGFsZ2FlIGFuZCAgZGVjbGluZSBvZiBtYWNyb3BoeXRlcyBsZWFkaW5nIHRvIGEgaGlnaC10dXJiaWRpdHkgc3RhdGUgb2Ygd2F0ZXIsIGluIHdoaWNoIGZldyBzcGVjaWVzIGNhbiAgc3Vydml2ZSAoU2NoZWZmZXIgZXQgYWwuIDE5OTMpLiBUbyBhZGRyZXNzIHRoZSBldXRyb3BoaWNhdGlvbiBhbmQgZXhjZXNzIG51dHJpZW50IGlzc3VlcyBpbiAgYXF1YXRpYyBlY29zeXN0ZW1zLCBwaHl0b3JlbWVkaWF0aW9uIG9mZmVycyBhIHByb21pc2luZyBzb2x1dGlvbiAoTGl1IGV0IGFsLiAyMDIxKS4gQXF1YXRpYyAgcGxhbnRzIGxpa2UgZHVja3dlZWRzLCB3YXRlciBsZXR0dWNlIChQaXN0aWEgc3RyYXRpb3RlcyksIGFuZCB3YXRlciBsaWxpZXMgaGF2ZSBkZW1vbnN0cmF0ZWQgIHRoZWlyIGFiaWxpdHkgdG8gY29tYmF0IGV1dHJvcGhpY2F0aW9uIGJ5IGFic29yYmluZyBhbmQgYXNzaW1pbGF0aW5nIGV4Y2VzcyBwaG9zcGhvcnVzIGFuZCAgbml0cm9nZW4gKE5haGFyICYgSG9xdWUgMjAyMSkuIEhvd2V2ZXIsIHRoZXNlIHBsYW50cyBjYW4gc29tZXRpbWVzIGdyb3cgdW5jb250cm9sbGFibHksICBwb3RlbnRpYWxseSBjb3ZlcmluZyB0aGUgd2F0ZXIgc3VyZmFjZXMuIFRvIGF2b2lkIHN1Y2ggc2NlbmFyaW9zLCByZWd1bGFyIG1vbml0b3JpbmcgaXMgZXNzZW50aWFsIChMaXUgZXQgYWwuIDIwMjEpLiBQcm9tb3RpbmcgcHVibGljIGF3YXJlbmVzcyBpcyB0aGUgY29ybmVyc3RvbmUgZm9yIG1vYmlsaXppbmcgc3VwcG9ydCBhbmQgZW5zdXJpbmcgZWZmZWN0aXZlIGV1dHJvcGhpY2F0aW9uIG1hbmFnZW1lbnQuIENvbnRyb2xsZWQgZXJhZGljYXRpb24gb2YgZXhjZXNzICBmbG9yYSB3aGVuIG5lY2Vzc2FyeSBpcyB2aXRhbCB0byBwcmV2ZW50IG92ZXJncm93dGguIFRoaXMgaW50ZWdyYXRlZCBhcHByb2FjaCBjYW4gZWZmZWN0aXZlbHkgIHJlZHVjZSBldXRyb3BoaWNhdGlvbiBpbiB0aGUgd2F0ZXIgYm9kaWVzIChXYW5nIGV0IGFsLiAyMDIyKS4gRWFjaCBzcGVjaWVzIG9mIHdldGxhbmQtZGVwZW5kZW50IGJpcmQgaGFzIGEgdW5pcXVlIGFuZCBjb21wbGV4IHNldCBvZiBuZWVkcyBmb3Igd2V0bGFuZCBoYWJpdGF0cyB0aGF0IG1ha2VzIGl0IGRpZmZpY3VsdCB0byBnZW5lcmFsaXplIGFib3V0IGhvdyBsb3NzIG9yIGRlZ3JhZGF0aW9uIG9mIHdldGxhbmRzIGFmZmVjdHMgYmlyZCBwb3B1bGF0aW9ucy4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NzkuMjk1NjM5MDM4MDg1OTQsImxlZnQiOjEwLjMwMzAwNjE3MjE4MDE3Niwid2lkdGgiOjc5Ljk1MjcyODI3MTQ4NDM4LCJoZWlnaHQiOjEuNzA5MTk3ODc4ODM3NTg1NH19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo4MS4yMTI1Mzk2NzI4NTE1NiwibGVmdCI6MTAuMzAzMDA2MTcyMTgwMTc2LCJ3aWR0aCI6NzkuOTU5MjgxOTIxMzg2NzIsImhlaWdodCI6MS43MDkxOTc4Nzg4Mzc1ODU0fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjgzLjEyOTQ0NzkzNzAxMTcyLCJsZWZ0IjoxMC4zMDMwMDYxNzIxODAxNzYsIndpZHRoIjo3OS45NDI3MjYxMzUyNTM5LCJoZWlnaHQiOjEuNzA5MTk3ODc4ODM3NTg1NH19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo4NS4wNDYzNDg1NzE3NzczNCwibGVmdCI6MTAuMzAzMDA4MDc5NTI4ODA5LCJ3aWR0aCI6NzkuOTM5NDUzMTI1LCJoZWlnaHQiOjEuNzA5MTk3ODc4ODM3NTg1NH19LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo4Ni45NjMyNTY4MzU5Mzc1LCJsZWZ0IjoxMC4zMDMwMTA5NDA1NTE3NTgsIndpZHRoIjo3OS45NDE4NDg3NTQ4ODI4MSwiaGVpZ2h0IjoxLjcwOTE5Nzg3ODgzNzU4NTR9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6ODguODgwMTY1MTAwMDk3NjYsImxlZnQiOjEwLjMwMzAxMzgwMTU3NDcwNywid2lkdGgiOjc5LjkzNzUsImhlaWdodCI6MS43MDkxOTc4Nzg4Mzc1ODU0fX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjkwLjc5NzA2NTczNDg2MzI4LCJsZWZ0IjoxMC4zMDMwMTM4MDE1NzQ3MDcsIndpZHRoIjo3OS45Mzk2NTE0ODkyNTc4MSwiaGVpZ2h0IjoxLjcwOTE5Nzg3ODgzNzU4NTR9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6OTIuNzEzOTczOTk5MDIzNDQsImxlZnQiOjEwLjMwMzAxMzgwMTU3NDcwNywid2lkdGgiOjc5Ljk1MDMyNTAxMjIwNzAzLCJoZWlnaHQiOjEuNzA5MTk3ODc4ODM3NTg1NH19LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjoxMi4yMDc5OTA2NDYzNjIzMDUsImxlZnQiOjEwLjMwMzAyNDI5MTk5MjE4OCwid2lkdGgiOjc5Ljk1MjUxNDY0ODQzNzUsImhlaWdodCI6Ny40NTk5MjI3OTA1MjczNDR9fSx7InBhZ2VOdW1iZXIiOjEzLCJiYm94Ijp7InRvcCI6MTkuODc1NjI1NjEwMzUxNTYyLCJsZWZ0IjoxMi4xMjEzMjQ1MzkxODQ1Nywid2lkdGgiOjc4LjExOTIyNDU0ODMzOTg0LCJoZWlnaHQiOjEuNzA5MTk3ODc4ODM3NTg1NH19LHsicGFnZU51bWJlciI6MTMsImJib3giOnsidG9wIjoyMS43OTI1MzM4NzQ1MTE3MiwibGVmdCI6MTAuMzAzMDI5MDYwMzYzNzcsIndpZHRoIjo3OS45NDkyNDkyNjc1NzgxMiwiaGVpZ2h0IjoxLjcwOTE5Nzg3ODgzNzU4NTR9fSx7InBhZ2VOdW1iZXIiOjEzLCJiYm94Ijp7InRvcCI6MjMuNzA5NDM2NDE2NjI1OTc3LCJsZWZ0IjoxMC4zMDMwMjkwNjAzNjM3Nywid2lkdGgiOjc5Ljk1MjI3MDUwNzgxMjUsImhlaWdodCI6MS43MDkxOTc4Nzg4Mzc1ODU0fX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiMjUifX1d. Restoration efforts like dredging failed without nutrient controls, as internal loading from anoxic sediments sustains cycles, exacerbated by prolonged ice-free periods from warming (Ji et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkRhb2JpbiIsImZhbWlseSI6IkppIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllhbnhpbmciLCJmYW1pbHkiOiJIYW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTGlhbmdob25nIiwiZmFtaWx5IjoiTG9uZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJYaWFva2FuZyIsImZhbWlseSI6IlhpbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJIdWl5YW4iLCJmYW1pbHkiOiJYdSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTaXFpYW4iLCJmYW1pbHkiOiJRaXUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSmlhbmdodWFpIiwiZmFtaWx5IjoiTWVuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJYaW5neGluZyIsImZhbWlseSI6IlpoYW8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiWWFuYW4iLCJmYW1pbHkiOiJIdWFuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJEZWZ1IiwiZmFtaWx5IjoiTGl1In19XSwiaWRzIjp7Imlzc24iOlsiMjI5Ni03MDFYIl0sImlzYm4iOltdLCJkb2kiOiIxMC4zMzg5L2Zldm8uMjAyMy4xMjk3MDQ3Iiwib3BlbmFsZXgiOiJXNDM5MDYxMDY2OCJ9LCJ0aXRsZSI6Ikh5cG94aWEgYW5kIGl0cyBmZWVkYmFjayByZXNwb25zZSB0byBhbGdhbCBibG9vbXMgYW5kIENINCBlbWlzc2lvbnMgaW4gc3VidHJvcGljYWwgcmVzZXJ2b2lycyIsImFic3RyYWN0IjoiQW50aHJvcG9nZW5pYyBudXRyaWVudCBpbnB1dCBzdGltdWxhdGVzIHN1cmZhY2UgZXV0cm9waGljYXRpb24gYW5kIGFsZ2FsIGJsb29tcyBhbmQgdGh1cyBleGFjZXJiYXRlcyBveHlnZW4gZGVwbGV0aW9uIGFuZCBoeXBveGlhIGluIGFxdWF0aWMgZWNvc3lzdGVtcy4gSG93ZXZlciwgZXhpc3Rpbmcgc3R1ZGllcyBoYXZlIHBhaWQgbGl0dGxlIGF0dGVudGlvbiBvbiB0aGUgdW5kZXJzdGFuZGluZyBvZiB0aGUgZmVlZGJhY2sgcmVsYXRpb25zaGlwcyBhbW9uZyBveHlnZW4gY29uc3VtcHRpb24sIGFsZ2FsIHN1Y2Nlc3Npb24gYW5kIGdyZWVuaG91c2UgZ2FzIGVmZmVjdCBhZnRlciByaXZlciBkYW1taW5nLCBwYXJ0aWN1bGFybHkgaW4gVGhyZWUgR29yZ2VzIFJlc2Vydm9pci4gVGhpcyBmaWVsZCB3b3JrIGludmVzdGlnYXRlZCB0aGUgdGhlcm1hbCBzdHJhdGlmaWNhdGlvbiwgd2F0ZXIgcXVhbGl0eSBhbmQgcGh5dG9wbGFua3RvbiBpbiBmb3VyIHRyaWJ1dGFyaWVzIChQZW5neGkgQmF5IChQWEIpLCBEYW5pbmcgQmF5IChETkIpLCBTaGVubm9uZyBCYXkgKFNOQiksIGFuZCBYaWFuZ3hpIEJheSAoWFhCKSkgb2YgdGhlIFRocmVlIEdvcmdlcyBSZXNlcnZvaXIgKFRHUikgaW4gc3ByaW5nIChBcHJpbCAyNS0yOCwgMjAyMiksIGFuZCBleHBsb3JlZCB0aGUgZHJpdmVycyBvZiBoeXBveGlhIGFuZCBpdHMgZmVlZGJhY2sgcmVzcG9uc2UgdG8gYWxnYWwgYmxvb21zIGFuZCBtZXRoYW5lIChDSCA0ICkgZW1pc3Npb25zLiBUaGUgcmVzdWx0cyBzaG93ZWQgdGhhdCBkdXJpbmcgdGhlIG9ic2VydmF0aW9uIHBlcmlvZCwgRE8sIHRoZXJtYWwgc3RyYXRpZmljYXRpb24sIGFuZCBhbGdhbCBibG9vbXMgd2VyZSBtb3JlIHNldmVyZSBpbiBQWEIuIFRoZSBoaWdoIENoZW1pY2FsIE94eWdlbiBEZW1hbmQgKENPRCBNbiApICgxMS4yNyBtZ%2BKAokwgLTEgKSBhbmQgRGlzc29sdmVkIE9yZ2FuaWMgQ2FyYm9uIChET0MpIGNvbnRlbnQgKDcuNTYgbWfigKJMIC0xICkgaW4gUFhCIHdlcmUgdGhlIG1haW4gY2F1c2VzIG9mIHRoZSBwcm9ncmVzc2l2ZSBoeXBveGlhLiBGdXJ0aGVybW9yZSwgc3Ryb25nZXIgZGVuc2l0eSBzdHJhdGlmaWNhdGlvbiBpbiBQWEIgaW5oaWJpdGVkIHRoZSB2ZXJ0aWNhbCBzdXBwbHkgb2YgRE8gYW5kIGFjY2VsZXJhdGVkIGJvdHRvbSBoeXBveGlhLiBCdXQgdGhlIGRvd25zbG9wZSBkZW5zaXR5IGN1cnJlbnQgcmVwbGVuaXNoZWQgYm90dG9tIERPIGluIHRoZSB1cHBlciByZWFjaGVzIG9mIHRoZSBETkIsIFNOQiwgYW5kIFhYQi4gVGhlIENINCBjb25jZW50cmF0aW9uIHdhcyBnZW5lcmFsbHkgaGlnaGVyIGluIGh5cG94aWEgem9uZSwgZXNwZWNpYWxseSBpbiB3aGVyZSBjeWFub2JhY3RlcmlhIGFyZSB0aGUgZG9taW5hbnQgc3BlY2llcyAoZWcuLCBQWDAzIGFuZCBQWDA0KS4gRGFtIGNvbnN0cnVjdGlvbiBsZWQgdG8gdGhlIHN1Y2Nlc3Npb24gb2YgYWxnYWwtZG9taW5hbnQgc3BlY2llcywgZXhlcnRpbmcgYSBzaWduaWZpY2FudCBpbmZsdWVuY2Ugb24gdGhlIHJpdmVyIGNhcmJvbiBjeWNsZS4gVGhlIGV4cGFuc2lvbiBvZiBjeWFub2JhY3RlcmlhbCBibG9vbXMgYW5kIHRoZSBjb25zZXF1ZW5jZSBvZiBtZXRhYm9saWMgc3Vic3RyYXRlcyBpbmNyZWFzZSBhcmUgZ3JhZHVhbGx5IGluY3JlYXNpbmcgb3h5Z2VuIGRlcHJpdmF0aW9uLCBldmVuIGh5cG94aWEsIGFuZCBlbmhhbmNpbmcgQ0ggNCBwcm9kdWN0aW9uIGluIHJpdmVycy4gVGhlc2UgZmluZGluZ3Mgc3VnZ2VzdCB0aGF0IHRoZSBzdWNjZXNzaW9uIG9mIGFsZ2FsLWRvbWluYW50IHNwZWNpZXMgY2F1c2VkIGJ5IHJpdmVyIGRhbW1pbmcgaGFzIGEgcHJvbm91bmNlZCBwb3NpdGl2ZSBmZWVkYmFjayBlZmZlY3Qgb24gcmVzZXJ2b2lyIGh5cG94aWEgcHJvY2Vzc2VzIGFuZCBncmVlbmhvdXNlIGVmZmVjdC4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMzM4OS9mZXZvLjIwMjMuMTI5NzA0NyIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cuZnJvbnRpZXJzaW4ub3JnL2FydGljbGVzLzEwLjMzODkvZmV2by4yMDIzLjEyOTcwNDcvcGRmP2lzUHVibGlzaGVkVjI9RmFsc2UiLCJpc3N1ZWREYXRlIjoiMjAyNC0wMS0wNSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZyb250aWVycyBpbiBFY29sb2d5IGFuZCBFdm9sdXRpb24iLCJ2b2x1bWUiOjExLCJpbXBhY3RGYWN0b3IiOiI0Ljc4In0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkZyb250aWVycyBNZWRpYSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NDAsImNpdGVkQnlDb3VudCI6NX19LCJxdW90ZSI6Im9nZW5pYyBudXRyaWVudCBpbnB1dCBzdGltdWxhdGVzIHN1cmZhY2UgZXV0cm9waGljYXRpb24gYW5kIGFsZ2FsIGJsb29tcyBhbmQgdGh1cyBleGFjZXJiYXRlcyBveHlnZW4gZGVwbGV0aW9uIGFuZCBoeXBveGlhIGluIGFxdWF0aWMgZWNvc3lzdGVtcy4gSG93ZXZlciwgZXhpc3Rpbmcgc3R1ZGllcyBoYXZlIHBhaWQgbGl0dGxlIGF0dGVudGlvbiBvbiB0aGUgdW5kZXJzdGFuZGluZyBvZiB0aGUgZmVlZGJhY2sgcmVsYXRpb25zaGlwcyBhbW9uZyBveHlnZW4gY29uc3VtcHRpb24sIGFsZ2FsIHN1Y2Nlc3Npb24gYW5kIGdyZWVuaG91c2UgZ2FzIGVmZmVjdCBhZnRlciByaXZlciBkYW1taW5nLCAuLi4gRE8gcGxheXMgYSBrZXkgcm9sZSBpbiB0aGUgYmlvZ2VvY2hlbWljYWwgY3ljbGUgb2Ygc3VyZmFjZSB3YXRlcnMgYW5kIGlzIGEgc2Vuc2l0aXZlIGluZGljYXRvciBvZiBwaHlzaWNhbCBhbmQgYmlvZ2VvY2hlbWljYWwgY2hhbmdlcyBpbiByZXNlcnZvaXIgYXF1YXRpYyBlY29zeXN0ZW1zICggV2V0emVsLCAyMDAxICkuIFdoZW4gd2F0ZXJzIGVudGVyIGFuIGFuYWVyb2JpYyBzdGF0ZSwgYXF1YXRpYyBvcmdhbmlzbXMgZGllIGluIGxhcmdlIG51bWJlcnMgYW5kIHRoZSByZWxlYXNlIG9mIHNlZGltZW50LXJlZHVjaW5nIHBvbGx1dGFudHMgKGUuZy4sIGFtbW9uaWEgbml0cm9nZW4sIC4uLiAyMDIyXSApLCBmdXJ0aGVyIGRldGVyaW9yYXRpbmcgd2F0ZXIgcXVhbGl0eS4gV2l0aCB0aGUgaW5jcmVhc2Ugb2YgcG9sbHV0aW9uIGxvYWRzIGluIHdhdGVyc2hlZHMgYW5kIHRoZSBpbnRlbnNpZmljYXRpb24gb2Ygd2F0ZXIgZXV0cm9waGljYXRpb24sIHdhdGVyIGh5cG94aWEgaGFzIGJlY29tZSBhIHNldmVyZSBnbG9iYWwgZWNvbG9naWNhbCBhbmQgZW52aXJvbm1lbnRhbCBwcm9ibGVtIGluIHJlc2Vydm9pcnMgKCBTdW4gZXTCoGFsLiwgMjAyMSA7IEphaXN3YWwgZXTCoGFsLiwgMjAyMyApLiBGb2N1c2luZyBvbiB0aGUgZHluYW1pYyBjaGFuZ2VzIGluIn1d. Community-led de-weeding provides temporary relief but underscores needs for integrated watershed management.

Chilika Lagoon, India's largest brackish wetland, illustrates HAB-fishery collapse amid hydrological alterations and aquaculture effluents. Cyanobacterial dominance post-2000 inlet breach caused hypoxia, decimating prawn and fish stocks yields dropped 40% in bloom seasons impacting 150,000 fishers' livelihoods (Jilbert et al., 2020; Nair & Nayak, 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRvbSIsImZhbWlseSI6IkppbGJlcnQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUmFvdWzigJBNYXJpZSIsImZhbWlseSI6IkNvdXR1cmUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQnJpYW4iLCJmYW1pbHkiOiJIdXNlciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLYWxldmkiLCJmYW1pbHkiOiJTYWxvbmVuIn19XSwiaWRzIjp7Imlzc24iOlsiMDAxOC04MTU4IiwiMTU3My01MTE3Il0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDA3L3MxMDc1MC0wMjAtMDQ0NTcteCIsIm1hZyI6IjMxMDA0NTkwNTAiLCJvcGVuYWxleCI6IlczMTAwNDU5MDUwIn0sInRpdGxlIjoiUHJlZmFjZTogUmVzdG9yYXRpb24gb2YgZXV0cm9waGljIGxha2VzOiBjdXJyZW50IHByYWN0aWNlcyBhbmQgZnV0dXJlIGNoYWxsZW5nZXMiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAwNy9zMTA3NTAtMDIwLTA0NDU3LXgiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vbGluay5zcHJpbmdlci5jb20vY29udGVudC9wZGYvMTAuMTAwNy9zMTA3NTAtMDIwLTA0NDU3LXgucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjAtMTEtMTYiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA3IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJIeWRyb2Jpb2xvZ2lhIiwiaXNzdWUiOjIxLCJ2b2x1bWUiOjg0NywiaW1wYWN0RmFjdG9yIjoiMy4wMCJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjQzNDMsImxhc3RQYWdlIjo0MzU3fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlNwcmluZ2VyIFNjaWVuY2UrQnVzaW5lc3MgTWVkaWEifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjk1LCJjaXRlZEJ5Q291bnQiOjU4fX0sInF1b3RlIjoiU2ltaWxhciBkZWdyYWRhdGlvbiBvZiBmcmVzaHdhdGVyIGVjb3N5c3RlbXMgaXMgb2JzZXJ2ZWQgd29ybGR3aWRlLCB3aXRoIHRoZSB0aW1lbGluZSBvZiBodW1hbiBwcmVzc3VyZSBkaWN0YXRlZCBieSB0cmVuZHMgaW4gdXJiYW5pemF0aW9uIGFuZCBpbnRlbnNpdmUgYWdyaWN1bHR1cmUgaW4gZGlmZmVyZW50IGdlb2dyYXBoaWMgcmVnaW9ucyAoZS5nLiwgVmVubmVtbyBldCBhbC4sIDIwMDkpLiBUaGUgcHJpbmNpcGFsIGRyaXZlciBvZiBlY29zeXN0ZW0gc3RyZXNzIGluIG1vc3QgYXF1YXRpYyBzeXN0ZW1zIGlzIGV1dHJvcGhpY2F0aW9uLCB0aGUgZXhjZXNzIGdyb3d0aCBvZiBhbGdhZSBkcml2ZW4gYnkgYW50aHJvcG9nZW5pYyBpbnB1dHMgb2YgdGhlIG1hY3JvbnV0cmllbnRzIG5pdHJvZ2VuIChOKSBhbmQgcGhvc3Bob3J1cyAoUCkgKFNtaXRoLCAyMDAzOyBCaXJrIGV0IGFsLiwgMjAyMCkuIE51dHJpZW50cyBhcmUgZW1pdHRlZCBmcm9tIGJvdGggcG9pbnQgYW5kIGRpZmZ1c2Ugc291cmNlcyBpbiB0aGUgaHVtYW4gZW52aXJvbm1lbnQgYW5kIGVudGVyIGFxdWF0aWMgc3lzdGVtcyB2aWEgc3VyZmFjZSBydW5vZmYsIGFzIHdlbGwgYXMgZ3JvdW5kLXdhdGVyIGFuZCBhdG1vc3BoZXJpYyBkZXBvc2l0aW9uIChQYWVybCwgMTk5NykuIEV1dHJvcGhpY2F0aW9uIG9mdGVuIGxlYWRzIHRvIHRoZSBkb21pbmFuY2Ugb2YgcHJpbWFyeSBwcm9kdWN0aW9uIGJ5IGN5YW5vYmFjdGVyaWEsIGluY2x1ZGluZyB0b3hpYyBnZW5lcmEgc3VjaCBhcyBEb2xpY2hvc3Blcm11bSwgQXBoYW5pem9tZW5vbiwgYW5kIE1pY3JvY3lzdGlzIChIdWlzbWFuIGV0IGFsLiwgMjAxOCksIGFzIHdlbGwgYXMgb3h5Z2VuIGRlcGxldGlvbiBhbmQgaGFiaXRhdCBkZWdyYWRhdGlvbiBpbiBsYWtlcyAoQXJlbmQgZXQgYWwuLCAyMDExKS4gQmVjYXVzZSBsYWtlcyBwcm92aWRlIGEgcmFuZ2Ugb2YgY3J1Y2lhbCBlY29zeXN0ZW0gc2VydmljZXMgKFdpbHNvbiAmIENhcnBlbnRlciwgMTk5OSksIHRoZXJlIGlzIGEgZ3Jvd2luZyBuZWVkIGZvciBtYW5hZ2VtZW50IGFwcHJvYWNoZXMgdGhhdCByZXN0b3JlIHdhdGVyIHF1YWxpdHkgaW4gZnJlc2h3YXRlciBzeXN0ZW1zLiBXaXRoaW4gdGhlIGZyYW1ld29yayBvZiBVbml0ZWQgTmF0aW9ucyBTdXN0YWluYWJsZSBEZXZlbG9wLW1lbnQgR29hbCA2IChDbGVhbiBXYXRlciBhbmQgU2FuaXRhdGlvbiksIHNldmVyYWwgZGlzdGluY3QgdGFyZ2V0cyByZWxhdGUgdG8gdGhlIGltcHJvdmVtZW50IG9mIHdhdGVyIHF1YWxpdHkgYW5kIHByb3RlY3Rpb24gb2YgYXF1YXRpYyBlY29zeXN0ZW1zIGJ5IDIwMzAgKFVuaXRlZCBOYXRpb25zLCAyMDE1KS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyOC4yOTg4NjQzNjQ2MjQwMjMsImxlZnQiOjguNTQ5MjA1NzgwMDI5Mjk3LCJ3aWR0aCI6MzkuMzcxNDk0MjkzMjEyODksImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MjkuOTkxMTM0NjQzNTU0Njg4LCJsZWZ0Ijo4LjU0OTIwNTc4MDAyOTI5Nywid2lkdGgiOjM5LjM2NTExOTkzNDA4MjAzLCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMxLjY4MzQwNjgyOTgzMzk4NCwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNjk2NjMyMzg1MjUzOSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozMy4zNjc5NzMzMjc2MzY3MiwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNzU2NzkwMTYxMTMyOCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNS4wNjAyNDkzMjg2MTMyOCwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNzA5NjQwNTAyOTI5NywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNi43NTI1MTc3MDAxOTUzMSwibGVmdCI6OC41NDkyMDg2NDEwNTIyNDYsIndpZHRoIjozOS4zNzY5NjA3NTQzOTQ1MywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC40MzcwODgwMTI2OTUzMSwibGVmdCI6OC41NDkyMDg2NDEwNTIyNDYsIndpZHRoIjozOS4zNjY1Njk1MTkwNDI5NywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0MC4xMjkzNTYzODQyNzczNDQsImxlZnQiOjguNTQ5MjA4NjQxMDUyMjQ2LCJ3aWR0aCI6MzkuMzY0Mzc5ODgyODEyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0MS44MjE2MjQ3NTU4NTkzNzUsImxlZnQiOjguNTQ5MjA4NjQxMDUyMjQ2LCJ3aWR0aCI6MzkuMzY1NjczMDY1MTg1NTUsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDMuNTA2MTk1MDY4MzU5Mzc1LCJsZWZ0Ijo4LjU0OTM4OTgzOTE3MjM2Mywid2lkdGgiOjM5LjM3MDM5MTg0NTcwMzEyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0NS4xOTg0NjM0Mzk5NDE0MDYsImxlZnQiOjguNTQ5NTcyOTQ0NjQxMTEzLCJ3aWR0aCI6MzkuMzY1ODQ0NzI2NTYyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ni44OTA3MzU2MjYyMjA3LCJsZWZ0Ijo4LjU0OTU3Mjk0NDY0MTExMywid2lkdGgiOjM5LjM3Mjk1NTMyMjI2NTYyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0OC41NzUzMDIxMjQwMjM0NCwibGVmdCI6OC41NDk1NzI5NDQ2NDExMTMsIndpZHRoIjozOS4zNzI0MjEyNjQ2NDg0NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1MC4yNjc1NzgxMjUsImxlZnQiOjguNTQ5NTc2NzU5MzM4Mzc5LCJ3aWR0aCI6MzkuMzcxMTI0MjY3NTc4MTI1LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjUxLjk1OTg0MjY4MTg4NDc2NiwibGVmdCI6OC41NDk1NzY3NTkzMzgzNzksIndpZHRoIjozOS4zNjc4MzU5OTg1MzUxNTYsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTMuNTc0MTE5NTY3ODcxMDk0LCJsZWZ0Ijo4LjU0OTU3Njc1OTMzODM3OSwid2lkdGgiOjM5LjM3MDc3NzEzMDEyNjk1LCJoZWlnaHQiOjEuMzM2ODk1NTg1MDYwMTE5Nn19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjU1LjI2NjM5MTc1NDE1MDM5LCJsZWZ0Ijo4LjU0OTc2MDgxODQ4MTQ0NSwid2lkdGgiOjM5LjM2NjU4MDk2MzEzNDc2NiwiaGVpZ2h0IjoxLjMzNjg5NTU4NTA2MDExOTZ9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1Ny4wMjg5NTM1NTIyNDYwOTQsImxlZnQiOjguNTQ5OTQ0ODc3NjI0NTEyLCJ3aWR0aCI6MzkuMzYyMDI2MjE0NTk5NjEsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTguNzEzNTIzODY0NzQ2MDk0LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjkuMjk5OTgyMDcwOTIyODUyLCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjYwLjQwNTc5MjIzNjMyODEyNSwibGVmdCI6MTAuNjIyNDU1NTk2OTIzODI4LCJ3aWR0aCI6MzcuMjg3ODY4NDk5NzU1ODYsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NjIuMDk4MDYwNjA3OTEwMTU2LCJsZWZ0Ijo4LjU0OTk0MzkyMzk1MDE5NSwid2lkdGgiOjM5LjM3MzMyNTM0NzkwMDM5LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjYzLjc4MjYzMDkyMDQxMDE1NiwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzMxMjY5ODM2NDI1OCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2NS40NzQ5MDY5MjEzODY3MiwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNjUxMTk5MzQwODIwMywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2Ny4xNjcxNzUyOTI5Njg3NSwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNjk0ODAxMzMwNTY2NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2OC43ODE0NDgzNjQyNTc4MSwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzczMzg0MDk0MjM4MywiaGVpZ2h0IjoxLjMzNjg5MTUzMTk0NDI3NX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjcwLjU0NDAxMzk3NzA1MDc4LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjM5LjM2OTMwODQ3MTY3OTY5LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjcyLjIzNjI4OTk3ODAyNzM0LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjM5LjM3MjAzMjE2NTUyNzM0NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo3My45MjA4NDUwMzE3MzgyOCwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzEzMDczNzMwNDY4NzUsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNDM0NCJ9fSx7Im9yaWdpbiI6NCwid29yayI6eyJ0eXBlIjo4LCJzdWJ0eXBlIjp7Im9uZW9mS2luZCI6ImFydGljbGVUeXBlIiwiYXJ0aWNsZVR5cGUiOjF9LCJjb250cmlidXRvcnMiOlt7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJOYXZ5YSBWaWtyYW1hbiIsImZhbWlseSI6Ik5haXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUHJhdGVlcCBLdW1hciIsImZhbWlseSI6Ik5heWFrIn19XSwiaWRzIjp7Imlzc24iOlsiMjI5Ni03NzQ1Il0sImlzYm4iOltdLCJkb2kiOiIxMC4zMzg5L2ZtYXJzLjIwMjMuMTA4NzI5NiIsIm9wZW5hbGV4IjoiVzQzODI3MjMxMzMifSwidGl0bGUiOiJVbmNvdmVyaW5nIHdhdGVyIHF1YWxpdHkgYW5kIGV2YWx1YXRpbmcgdnVsbmVyYWJpbGl0aWVzIG9mIHNtYWxsLXNjYWxlIGZpc2hlcmllcyBpbiBDaGlsaWthIExhZ29vbiwgSW5kaWEiLCJhYnN0cmFjdCI6IlNtYWxsLXNjYWxlIGZpc2hlcmllcyAoU1NGcykgcGxheSBhIHN0cm9uZyByb2xlIGluIHN1c3RhaW5pbmcgbWlsbGlvbnMgb2YgbGl2ZWxpaG9vZHMsIGZvb2Qgc2VjdXJpdHksIG51dHJpdGlvbiwgYW5kIGluY29tZSBnbG9iYWxseSBidXQgdGhlIGZpc2hlcnMgZW5nYWdlZCBpbiB0aGlzIHNlY3RvciBzaW11bHRhbmVvdXNseSBleHBlcmllbmNlIGhpZ2ggbGV2ZWxzIG9mIHZ1bG5lcmFiaWxpdHkgYW5kIHByb2Nlc3NlcyBvZiBtYXJnaW5hbGlzYXRpb24uIFNldmVyYWwgZmFjdG9ycyBhcmUgYXR0cmlidXRlZCB0byB0aGUgbXVsdGlkaW1lbnNpb25hbCB2dWxuZXJhYmlsaXRpZXMgdGhlIHNtYWxsLXNjYWxlIGZpc2hlcnMgZXhwZXJpZW5jZSwgc3Bhbm5pbmcgYm90aCBuYXR1cmFsIChlLmcuLCBuYXR1cmFsIGRpc2FzdGVycywgZWNvc3lzdGVtIGNoYW5nZSkgYW5kIGFudGhyb3BvZ2VuaWMgKGUuZy4sIHBvbGljeSBjaGFuZ2UsIGh5ZHJvbG9naWNhbCBpbnRlcnZlbnRpb25zLCBhcXVhY3VsdHVyZSkgcHJlc3N1cmVzLiBXaGlsZSB0aGVyZSBpcyBtdWNoIGxpdGVyYXR1cmUgb24gdmFyaW91cyBuYXR1cmFsIGFuZCBodW1hbiBkcml2ZXJzIG9mIHZ1bG5lcmFiaWxpdHkgaW4gc21hbGwtc2NhbGUgZmlzaGVyeSBjb21tdW5pdGllcywgYW4gYWJzZW5jZSBvZiByZXNlYXJjaCBjb25uZWN0aW5nIHZ1bG5lcmFiaWxpdHkgd2l0aCB3YXRlciBxdWFsaXR5IGlzIGV2aWRlbnQuIEZpc2hlciBjb21tdW5pdGllcyBvZnRlbiB0YWxrIGFib3V0IGZpc2ggaW4gcmVsYXRpb24gdG8gdGhlIGhlYWx0aCBvZiB0aGVpciBhcXVhdGljIGhhYml0YXRzIHdoZXJlaW4gd2F0ZXIgcXVhbGl0eSBpcyBzZWVuIGFzIGEga2V5IHBhcmFtZXRlci4gVGhlIGxpbmsgYmV0d2VlbiBoZWFsdGh5IGZpc2ggYW5kIGdvb2QgcXVhbGl0eSB3YXRlciBoYXMgc2lnbmlmaWNhbnQgaW1wbGljYXRpb25zIGZvciBzdHJvbmcgYW5kIHZpYWJsZSBmaXNoaW5nIGNvbW11bml0aWVzLiBUaGlzIHBhcGVyIGV4YW1pbmVzIHRoZXNlIGxpbmtzIGZ1cnRoZXIgYnkgZm9jdXNpbmcgb24gdGhlIG5hdHVyZSBvZiB2dWxuZXJhYmlsaXRpZXMgY2F1c2VkIGJ5IHdhdGVyIHF1YWxpdHkgY2hhbmdlcyBpbiB0aGUgc21hbGwtc2NhbGUgZmlzaGVyeSBzeXN0ZW0gb2YgQ2hpbGlrYSBMYWdvb24gaW4gSW5kaWEuIFdlIHVuZGVydGFrZSBkZXRhaWxlZCBhbmFseXNpcyBvZiB0aGUgaW52YXNpdmUgc2hyaW1wIGFxdWFjdWx0dXJlIGFjdGl2aXRpZXMgYW5kIGh5ZHJvbG9naWNhbCBpbnRlcnZlbnRpb25zIGZvciBvcGVuaW5nIG9mIGEgbGFnb29uIGlubGV0IHdpdGggdGhlIEJheSBvZiBCZW5nYWwgYXMgdHdvIGRvbWluYW50IGRyaXZlcnMgYWR2ZXJzZWx5IGltcGFjdGluZyB3YXRlciBxdWFsaXR5IGFuZCBpbmNyZWFzaW5nIHZ1bG5lcmFiaWxpdGllcyBvZiB0aGUgZW50aXJlIHNtYWxsLXNjYWxlIGZpc2hlcmllcyBzb2NpYWwtZWNvbG9naWNhbCBzeXN0ZW0uIE91ciBhbmFseXNpcyBzdWdnZXN0cyB0aGF0IHRoZXJlIGFyZSBzdHJvbmcgaW50ZXJjb25uZWN0aW9ucyBiZXR3ZWVuIGNoYW5nZXMgaW4gd2F0ZXIgcXVhbGl0eSBhbmQgdGhlIGxldmVscyBvZiB2dWxuZXJhYmlsaXRpZXMgaW4gdGhlIFNTRnMgb2YgQ2hpbGlrYSBMYWdvb24uIFBvbGx1dGFudHMgc3VjaCBhcyBwZXN0aWNpZGVzLCBhbmQgb3JnYW5pYyBjb21wb3VuZHMgYWNjdW11bGF0ZSBpbiBmaXNoIHRpc3N1ZXMgYW5kIGFmZmVjdCB0aGVpciBncm93dGgsIHJlcHJvZHVjdGlvbiwgYW5kIG92ZXJhbGwgaGVhbHRoLiBUaGlzIGxlZCB0byBkZWNsaW5lcyBpbiBmaXNoIHBvcHVsYXRpb25zLCBtYWtpbmcgaXQgbW9yZSBkaWZmaWN1bHQgZm9yIGZpc2hlcnMgdG8gbWFrZSBhIGxpdmluZy4gSW4gYWRkaXRpb24gdG8gZGlyZWN0IGltcGFjdHMgb24gZmlzaCBwb3B1bGF0aW9ucywgcG9vciB3YXRlciBxdWFsaXR5IGFsc28gaGFzIGluZGlyZWN0IGVmZmVjdHMgb24gdGhlIHNvY2lhbCBhbmQgZWNvbm9taWMgdnVsbmVyYWJpbGl0eSBvZiBTU0ZzLiBGb3IgZXhhbXBsZSwgY29udGFtaW5hdGlvbiBvZiB3YXRlciBzb3VyY2VzIGxlZCB0byB0aGUgcmVkdWNlZCBudW1iZXIgb2YgZmlzaCBzcGVjaWVzIHJlZHVjaW5nIHRoZSBhbW91bnQgb2YgdGltZSBmaXNoZXJzIGNhbiBzcGVuZCBvbiBmaXNoaW5nIGFjdGl2aXRpZXMuIFRoaXMgYWxzbyBhZmZlY3RlZCB0aGUgbWFya2V0YWJpbGl0eSBvZiBmaXNoIHByb2R1Y3RzLCByZWR1Y2VkIGluY29tZSBhbmQgaW5jcmVhc2VkIHBvdmVydHkuIFRvIGZ1bGx5IHVuZGVyc3RhbmQgdGhlIGludGVyY29ubmVjdGlvbnMgYmV0d2VlbiB3YXRlciBxdWFsaXR5IGFuZCB2dWxuZXJhYmlsaXR5IGluIFNTRnMgaW4gQ2hpbGlrYSBMYWdvb24sIGl0IGlzIGltcG9ydGFudCB0byBjb25zaWRlciBib3RoIGVudmlyb25tZW50YWwgYW5kIHNvY2lhbCBmYWN0b3JzLCBhcyB3ZWxsIGFzIHRoZSBjb21wbGV4IGZlZWRiYWNrIGxvb3BzIGJldHdlZW4gdGhlc2UgZmFjdG9ycy4gVGhlIHN0dWR5IGhlbHBzIGluIGJyaWRnaW5nIGEgY3J1Y2lhbCBnYXAgaW4gb3VyIHVuZGVyc3RhbmRpbmcgb2YgdGhlIHJvbGUgb2Ygd2F0ZXIgcXVhbGl0eSBpbiB2dWxuZXJhYmlsaXR5IGFuYWx5c2lzIHdpdGhpbiByZXNvdXJjZSBkZXBlbmRlbnQgY29tbXVuaXRpZXMuIFdlIGNvbmNsdWRlIHdpdGgga2V5IGluc2lnaHRzIG9uIHBvc3NpYmxlIGNvcGluZyByZXNwb25zZXMgYW5kIGFkYXB0aXZlIGNhcGFjaXR5IG5lY2Vzc2FyeSBmb3IgdGhlIHNtYWxsLXNjYWxlIGZpc2hlcmllcyBjb21tdW5pdGllcyB0byB0cmFuc2l0aW9uIHRvd2FyZCB2aWFiaWxpdHkuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjMzODkvZm1hcnMuMjAyMy4xMDg3Mjk2IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5mcm9udGllcnNpbi5vcmcvYXJ0aWNsZXMvMTAuMzM4OS9mbWFycy4yMDIzLjEwODcyOTYvcGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjMtMDYtMjkiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTEwIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJGcm9udGllcnMgaW4gTWFyaW5lIFNjaWVuY2UiLCJ2b2x1bWUiOjEwLCJpbXBhY3RGYWN0b3IiOiI0LjI4In0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkZyb250aWVycyBNZWRpYSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NTEsImNpdGVkQnlDb3VudCI6NX19LCJxdW90ZSI6IldhdGVyIHF1YWxpdHkgb2YgQ2hpbGlrYSBMYWdvb24gaGFzIGRlZ3JhZGVkIGR1ZSB0byBhbnRocm9wb2dlbmljIGludGVydmVudGlvbnMgYW5kIG5hdHVyYWwgZm9yY2VzIG9mIGNoYW5nZS4gVmFyaW91cyBhY3Rpdml0aWVzIGluIHRoZSBDaGlsaWthIExhZ29vbiwgc3VjaCBhcyBpbmR1c3RyaWFsIHdhc3Rld2F0ZXIgZGlzcG9zYWwsIHNld2FnZSBkdW1waW5nLCBhcXVhY3VsdHVyZSwgaHlkcm9sb2dpY2FsIGludGVydmVudGlvbnMsIGFuZCBjeWNsb25lcywgYXJlIHByb2R1Y2luZyBzYWxpbml0eSBjaGFuZ2VzLCBzZWRpbWVudCBkZXBvc2l0aW9uLCBudXRyaWVudCBlbnJpY2htZW50LCBldXRyb3BoaWNhdGlvbiwgYW5kIGRlYWQgem9uZXMsIGFzIHN0YXRlZC4gRmlzaCBzdXJ2aXZhbCBpcyBhdCByaXNrIGFzIGEgcmVzdWx0IG9mIHRoZSBzaWduaWZpY2FudCByZWR1Y3Rpb24gaW4gZW52aXJvbm1lbnRhbCBjaXJjdW1zdGFuY2VzLCB3aGljaCBkaW1pbmlzaGVzIGZpc2hlcm1lbuKAmXMgaW5jb21lIGFuZCBwdXNoZXMgdGhlbSBvdXQgb2Ygc29jaWV0eS4gVGhlc2UgY2F1c2VzIGFmZmVjdGluZyB0aGUgd2F0ZXIgcXVhbGl0eSBvZiB0aGUgTGFnb29uIGRlcGljdCBtYW55IGZhY2VzIG9mIHZ1bG5lcmFiaWxpdHkgaW4gU1NGIGNvbW11bml0aWVzLiBBcyB0aGUgQ2hpbGlrYSBMYWdvb24gZW52aXJvbm1lbnQgaXMgZXhwZWN0ZWQgdG8gdW5kZXJnbyBpbmNyZWFzaW5nbHkgZHJhc3RpYyBjaGFuZ2VzIChzdWNoIGFzIGNhdGNoIGNvbXBvc2l0aW9uLCBjYXRjaCBjYXBhY2l0eSwgYmlvZGl2ZXJzaXR5IHZhcmlhdGlvbnMsIHdhdGVyIHF1YWxpdHksIGFuZCBmaXNoZXJ5IHJldmVudWVzKSwgaW50ZXJuYXRpb25hbCBwb2xpY3kgaGFzIGJlY29tZSBtb3JlIGludGVyZXN0ZWQgaW4gYWRhcHRhdGlvbiBtZWFzdXJlcy4gR2l2ZW4gaG93IGRpdmVyc2UgU1NGIGVjb3N5c3RlbXMgYXJlIGV4cGVjdGVkIHRvIGJlIGJldHdlZW4gY29udGluZW50cywgdGhlcmUgaXMgbm8g4oCcb25lIHNpemUgZml0cyBhbGzigJ0gYXBwcm9hY2guIFRoZSBwYXBlciBvdXRsaW5lZCBzZXZlcmFsIGNvcGluZyBhbmQgYWRhcHRhdGlvbiBtZWNoYW5pc21zIHRvIGltcHJvdmUgd2F0ZXIgcXVhbGl0eSBhbmQgY29udGludWUgZmlzaGluZyBpbiBhIHN1c3RhaW5hYmxlIHdheS4gQWRhcHRhdGlvbiBzdHJhdGVnaWVzIGNhbiBvY2Nhc2lvbmFsbHkgaGF2ZSB1bmZvcmVzZWVuIGNvbnNlcXVlbmNlcyBmb3IgZmlzaGVyaWVzIGFuZCBmaXNoaW5nIGNvbW11bml0aWVzLiBBbnkgdW5leHBlY3RlZCBjb25zZXF1ZW5jZXMgb2YgYWRhcHRhdGlvbiBjYW4gYmUgbWl0aWdhdGVkIHRocm91Z2ggbG9uZy10ZXJtIHBsYW5uaW5nIGFuZCBpZGVudGlmaWNhdGlvbiBvZiBmdXR1cmUgb3B0aW9ucyB0aHJvdWdoIHNjZW5hcmlvIGFuYWx5c2lzLiBUaGUgZmluZGluZ3Mgb2YgdGhpcyBwYXBlciBjYW4gYmUgdXRpbGl6ZWQgdG8gZGV2ZWxvcCBwb2xpY2llcyB0byBwcm90ZWN0IHdhdGVyIHF1YWxpdHkgYW5kIFNTRnMgaW4gYSB2YXJpZXR5IG9mIGNvYXN0YWwgbGFnb29uIHNldHRpbmdzLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyMi4yODUwMzYwODcwMzYxMzMsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40OTMwNTM0MzYyNzkzLCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyMy43NzM1MTk1MTU5OTEyMSwibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ4MzM2NDEwNTIyNDYxLCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyNS4yNTU1ODQ3MTY3OTY4NzUsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40ODk1NTkxNzM1ODM5ODQsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjI2Ljc0NDA2ODE0NTc1MTk1MywibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjUwNjgwNTQxOTkyMTg3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjguMjI2MTMzMzQ2NTU3NjE3LCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDkwNDYzMjU2ODM1OTQsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjI5LjcxNDYxNjc3NTUxMjY5NSwibGVmdCI6OC4zMzE1OTI1NTk4MTQ0NTMsIndpZHRoIjo0MC40OTQwOTg2NjMzMzAwOCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MzEuMTk2NjgzODgzNjY2OTkyLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDk0MzkyMzk1MDE5NTMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjMyLjY4NTE2NTQwNTI3MzQ0LCJsZWZ0Ijo4LjMzMTU5NjM3NDUxMTcxOSwid2lkdGgiOjQwLjQ4NzQ0NTgzMTI5ODgzLCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozNC4xNjcyMzI1MTM0Mjc3MzQsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDk3NzQ1NTEzOTE2MDE2LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozNS42NTU3MTU5NDIzODI4MSwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40OTg2MzgxNTMwNzYxNywiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MzcuMTM3NzgzMDUwNTM3MTEsImxlZnQiOjEwLjY4NTE4NjM4NjEwODM5OCwid2lkdGgiOjM4LjE0MTEwNTY1MTg1NTQ3LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozOC42MjYyNjY0Nzk0OTIxOSwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40ODcxNTIwOTk2MDkzNzUsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQwLjEwODMzMzU4NzY0NjQ4NCwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40ODk4NTY3MTk5NzA3LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo0MS41OTY4MTMyMDE5MDQzLCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4ODY2NjUzNDQyMzgzLCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo0My4wNzg4ODAzMTAwNTg1OTQsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDgxODYxMTE0NTAxOTUsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQ0LjU2NzM2MzczOTAxMzY3LCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4MzA1ODkyOTQ0MzM2LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo0Ni4wNDk0MzA4NDcxNjc5NywibGVmdCI6OC4zMzMxMDg5MDE5Nzc1MzksIndpZHRoIjo0MC41MDA3NDc2ODA2NjQwNiwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDcuNTM3OTE0Mjc2MTIzMDUsImxlZnQiOjguMzMzMTA4OTAxOTc3NTM5LCJ3aWR0aCI6NDAuNDk1NjA1NDY4NzUsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQ5LjAxOTk4MTM4NDI3NzM0NCwibGVmdCI6OC4zMzMxMDYwNDA5NTQ1OSwid2lkdGgiOjQwLjUwNjgxNjg2NDAxMzY3LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo1MC41MDg0NjQ4MTMyMzI0MiwibGVmdCI6OC4zMzMxMDYwNDA5NTQ1OSwid2lkdGgiOjQwLjQ4ODUwNjMxNzEzODY3LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo1MS45OTA1MjgxMDY2ODk0NSwibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ5NzcyNjQ0MDQyOTY5LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo1My40NzkwMTE1MzU2NDQ1MywibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjMzLjE1OTQ0NjcxNjMwODU5NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NTQuOTYxMDc4NjQzNzk4ODMsImxlZnQiOjEwLjY4MzY3MDA0Mzk0NTMxMiwid2lkdGgiOjM4LjEzNjU5NjY3OTY4NzUsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjU2LjQ0OTU2MjA3Mjc1MzkwNiwibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ4NzE0NDQ3MDIxNDg0NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIxMyJ9fV0%3D. Microcystins in water and sediments bioaccumulate in shellfish, linking to cattle and human health incidents, with hospital admissions surging 30% (Gobler et al., 2024; Kumar & Mahajan, 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlBhd2FuIiwiZmFtaWx5IjoiS3VtYXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW1icmlzaCBLdW1hciIsImZhbWlseSI6Ik1haGFqYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyNTIzLTM5NjMiLCIyNTIzLTM5NzEiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMDcvczQyNDUyLTAyMC0zMDgyLTgiLCJtYWciOiIzMDM1OTYwMjI4Iiwib3BlbmFsZXgiOiJXMzAzNTk2MDIyOCJ9LCJ0aXRsZSI6IlRyb3BoaWMgc3RhdHVzIGFuZCBpdHMgcmVndWxhdGluZyBmYWN0b3IgZGV0ZXJtaW5hdGlvbiBhdCB0aGUgUmV3YWxzYXIgTGFrZSwgbm9ydGh3ZXN0IEhpbWFsYXlhIChIUCksIEluZGlhLCBiYXNlZCBvbiBzZWxlY3RlZCBwYXJhbWV0ZXJzIGFuZCBtdWx0aXZhcmlhdGUgc3RhdGlzdGljYWwgYW5hbHlzaXMiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAwNy9zNDI0NTItMDIwLTMwODItOCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9saW5rLnNwcmluZ2VyLmNvbS9jb250ZW50L3BkZi8xMC4xMDA3L3M0MjQ1Mi0wMjAtMzA4Mi04LnBkZiIsImlzc3VlZERhdGUiOiIyMDIwLTA2LTIyIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiU04gQXBwbGllZCBTY2llbmNlcyIsImlzc3VlIjo3LCJ2b2x1bWUiOjIsImltcGFjdEZhY3RvciI6IjcuMzUifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiU3ByaW5nZXIgTmF0dXJlIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo0MCwiY2l0ZWRCeUNvdW50IjoxOX19LCJxdW90ZSI6IkNoYW5naW5nIHF1YWxpdHkgaGFzIGNyZWF0ZWQgdGhlIHdhdGVyIHNjYXJjaXR5IGFuZCAgYSBodWdlIG1hc3Mgb2YgcG9wdWxhdGlvbiB0aHVzIGJlZW4gZGVwcml2ZWQgZnJvbSB1c2luZyAgdGhlIGZyZXNod2F0ZXIgZm9yIHZhcmlvdXMgcHVycG9zZXMgWzMsIDRdLiBIb3dldmVyLCB0aGUgIG9ibm94aW91cyBtYXRlcmlhbHMgdGhhdCBzdGF5IGluIHdhdGVyIGJvZGllcyBmb3IgYSBsb25nLXRpbWUgcGVyaW9kIGJyaW5nIGNoYW5nZXMgaW4gdGhlIHBoeXNpb2NoZW1pY2FsIGNoYXItYWN0ZXJpc3RpY3MgYXMgd2VsbCBhcyB0aGUgbmF0dXJhbCBzY2VuYXJpbyBvZiB0aGUgbGFrZXMgWzVdLiAgVGhlIEhpbWFsYXlhbiBsYWtlcyBhcmUgdW5kZXIgc3RyZXNzIGR1ZSB0byBtaXNtYW5hZ2VkIGh1bWFuIGFjdGl2aXRpZXMsIHdoaWNoIGluY3JlYXNlIG51dHJpZW50cyBhbmQgcG9sbHV0aW9uIGluIHRlcm1zIG9mIGhlYXZ5IG1ldGFscyB0b3hpY2l0eSBhbmQgYmFjdGVyaWFsIGNvbnRhbWluYXRpb24gIFs24oCTOF0uIFRoZSBtYW5hZ2VtZW50IGFuZCBwcmVzZXJ2YXRpb24gb2YgdGhlIGxha2VzIGF0ICBwcmVzZW50IGhhdmUgYmVjb21lIGEgZ3JlYXQgY2hhbGxlbmdlIGZvciBlbnZpcm9ubWVudGFsLWlzdHMgYW5kIG1hbmFnZXJzIFs1LCA5XS4gVGhlIGV1dHJvcGhpY2F0aW9uIGlzIGFsc28gYSBtYWluIHByb2JsZW0gb2YgdGhlICBmcmVzaHdhdGVyIGxha2VzLCBhbmQgaXQgaXMgc3BlY2lmaWVkIGZvciB0aGUgbGFrZXMgaGF2aW5nICBudXRyaWVudCBlbnJpY2htZW50LiBJdCBjYXVzZXMgZXhjZXNzaXZlIGdyb3d0aCBvZiBhbGdhZSAgYW5kIGlzIGNvbnNpZGVyZWQgdG8gYmUgcmVzcG9uc2libGUgZm9yIGRlZ3JhZGluZyB0aGUgIHF1YWxpdGF0aXZlIG5hdHVyZSBvZiB0aGUgbGFrZXMgWzUsIDEwXS4gVGhlIGV1dHJvcGhpY2F0aW9uICBpcyBmb2xsb3dlZCBieSB0aGUgYWxnYWwgYmxvb20sIHdoaWNoIGdlbmVyYXRlcyB0b3hpbiwgIHdvcnNlbnMgd2F0ZXIgcXVhbGl0eSwgZGVwbGV0ZXMgZGlzc29sdmVkIG94eWdlbiAoRE8pICogUGF3YW4gS3VtYXIsIHBhd2FuLmV2c0BnbWFpbC5jb20gfCAxRGVwYXJ0bWVudCBvZiBFbnZpcm9ubWVudGFsIFNjaWVuY2VzLCBDZW50cmFsIFVuaXZlcnNpdHkgb2YgSGltYWNoYWwgUHJhZGVzaCwgRGhhcmFtc2hhbGEgMTc2MjE1LCBJbmRpYS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2OC41Nzg0NjA2OTMzNTkzOCwibGVmdCI6OC41NzE0MTg3NjIyMDcwMzEsIndpZHRoIjo0MC4zMjc1MTQ2NDg0Mzc1LCJoZWlnaHQiOjEuNTE5ODU1MjYwODQ4OTk5fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NzAuMTU4OTk2NTgyMDMxMjUsImxlZnQiOjguNTY5NzM4Mzg4MDYxNTIzLCJ3aWR0aCI6NDAuMzU0NzI0ODg0MDMzMiwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjcxLjczOTU0NzcyOTQ5MjE5LCJsZWZ0Ijo4LjU2OTczODM4ODA2MTUyMywid2lkdGgiOjQwLjM0OTY5MzI5ODMzOTg0NCwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjczLjMyMDA5MTI0NzU1ODYsImxlZnQiOjguNTY4MDU5OTIxMjY0NjQ4LCJ3aWR0aCI6NDAuMDIwMDg4MTk1ODAwNzgsImhlaWdodCI6MS41MTk4NTUyNjA4NDg5OTl9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo3NC45MDA2NDIzOTUwMTk1MywibGVmdCI6OC41NjgwNTk5MjEyNjQ2NDgsIndpZHRoIjozOS45OTk5Mjc1MjA3NTE5NSwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjc2LjQ4MTE4NTkxMzA4NTk0LCJsZWZ0Ijo4LjU2ODA1NzA2MDI0MTcsIndpZHRoIjo0MC4zNTcwNzQ3Mzc1NDg4MywiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjc4LjA2MTcyOTQzMTE1MjM0LCJsZWZ0Ijo4LjU2OTczNjQ4MDcxMjg5LCJ3aWR0aCI6NDAuMzUzNzEzOTg5MjU3ODEsImhlaWdodCI6MS41MTk4NTUyNjA4NDg5OTl9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2MC42NzU3Mjc4NDQyMzgyOCwibGVmdCI6NTEuNDI3MTY1OTg1MTA3NDIsIndpZHRoIjo0MC4zNzg1NjY3NDE5NDMzNiwiaGVpZ2h0IjoxOC45MDU4NTUxNzg4MzMwMDh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo4MS45MjczMzc2NDY0ODQzOCwibGVmdCI6OC40MDE0OTc4NDA4ODEzNDgsIndpZHRoIjo3My45NTQ2OTY2NTUyNzM0NCwiaGVpZ2h0IjoyLjY3ODc2MjY3NDMzMTY2NX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEifX0seyJvcmlnaW4iOjUsIndvcmsiOnsidHlwZSI6OCwic3VidHlwZSI6eyJvbmVvZktpbmQiOiJhcnRpY2xlVHlwZSIsImFydGljbGVUeXBlIjoxfSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQ2hyaXN0b3BoZXIgSi4iLCJmYW1pbHkiOiJHb2JsZXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUnV0aCBXLiIsImZhbWlseSI6IkRyaW5rd2F0ZXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQS4gVy4iLCJmYW1pbHkiOiJBbnRob255In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikplbm5pZmVyIEEuIiwiZmFtaWx5IjoiR29sZXNraSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBbm4gTWFyaWUgRS4iLCJmYW1pbHkiOiJGYW11bGFyby1QZWNvcmEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTWljaGFlbCBCLiIsImZhbWlseSI6IldhbGxhY2UifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTm9yYSBSLiBXLiIsImZhbWlseSI6IlN0cmFxdWFkaW5lIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlJvbm9qb3kiLCJmYW1pbHkiOiJIZW0ifX1dLCJpZHMiOnsiaXNzbiI6WyIxNjY0LTMwMlgiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzODkvZm1pY2IuMjAyNC4xNDY0Njg2IiwicG1pZCI6IjM5NTY4OTk3Iiwib3BlbmFsZXgiOiJXNDQwNDEyMDk1MiIsImV4YSI6Imh0dHBzOi8vd3d3LmZyb250aWVyc2luLm9yZy9qb3VybmFscy9taWNyb2Jpb2xvZ3kvYXJ0aWNsZXMvMTAuMzM4OS9mbWljYi4yMDI0LjE0NjQ2ODYvZnVsbCJ9LCJ0aXRsZSI6IlNld2FnZS1hbmQgZmVydGlsaXplci1kZXJpdmVkIG51dHJpZW50cyBhbHRlciB0aGUgaW50ZW5zaXR5LCBkaXZlcnNpdHksIGFuZCB0b3hpY2l0eSBvZiBoYXJtZnVsIGN5YW5vYmFjdGVyaWFsIGJsb29tcyBpbiBldXRyb3BoaWMgbGFrZXMiLCJhYnN0cmFjdCI6IkN5YW5vYmFjdGVyaWFsIGhhcm1mdWwgYWxnYWwgYmxvb21zIChDSEFCcykgYXJlIHByb21vdGVkIGJ5IGV4Y2Vzc2l2ZSBudXRyaWVudCBsb2FkaW5nIGFuZCwgd2hpbGUgZmVydGlsaXplcnMgYW5kIHNld2FnZSBhcmUgdGhlIG1vc3QgcHJldmFsZW50IGV4dGVybmFsIG51dHJpZW50IHNvdXJjZXMgaW4gbW9zdCB3YXRlcnNoZWRzLCB0aGUgZGlmZmVyZW50aWFsIGVmZmVjdHMgb2YgdGhlc2UgbnV0cmllbnQgc291cmNlcyBvbiBDSEFCcyBhcmUgdW5rbm93bi4gSGVyZSwgd2UgdHJhY2tlZCBDSEFCcyBhbmQgcGVyZm9ybWVkIGV4cGVyaW1lbnRzIGluIGZpdmUgZGlzdGluY3QgbGFrZXMgYWNyb3NzIHRoZSBOb3J0aGVybiBVUyBpbmNsdWRpbmcgTGFrZSBFcmllLiBGZXJ0aWxpemVycyB3aXRoIGFtbW9uaXVtIGFuZCBvcnRob3Bob3NwaGF0ZSwgbWVtYnJhbmUgKDAuMiDOvG0pLWZpbHRlcmVkIHNld2FnZSAoZG9taW5hdGVkIGJ5IHJlZHVjZWQgZm9ybXMgb2Ygbml0cm9nZW4pIHNhbmQtYW5kIG1lbWJyYW5lLWZpbHRlcmVkIHNld2FnZSAoZG9taW5hdGVkIGJ5IG5pdHJhdGUpLCBhbmQgYW4gaW5vcmdhbmljIG51dHJpZW50IHNvbHV0aW9uIG9mIGFtbW9uaXVtIGFuZCBvcnRob3Bob3NwaGF0ZSB3ZXJlIHVzZWQgYXMgZXhwZXJpbWVudGFsIG51dHJpZW50IHNvdXJjZXMgZm9yIENIQUJzIGF0IE4tZXF1aXZhbGVudCwgZW52aXJvbm1lbnRhbGx5IHJlYWxpc3RpYyBjb25jZW50cmF0aW9ucy4gUGh5dG9wbGFua3RvbiBjb21tdW5pdGllcyB3ZXJlIGV2YWx1YXRlZCBmbHVvcm9tZXRyaWNhbGx5LCBtaWNyb3Njb3BpY2FsbHksIGFuZCB2aWEgaGlnaCB0aHJvdWdocHV0IHNlcXVlbmNpbmcgb2YgdGhlIDE2UyByUk5BIGdlbmUsIGFuZCBsZXZlbHMgb2YgbWljcm9jeXN0aW4gYW5kIHRoZSDOtCAxNSBOIGNvbnRlbnQgb2YgcGFydGljdWxhdGUgb3JnYW5pYyBuaXRyb2dlbiAozrRQTyAxNSBOKSB3ZXJlIHF1YW50aWZpZWQuIEZlcnRpbGl6ZXIgYW5kIGJvdGggc291cmNlcyBvZiB3YXN0ZXdhdGVyIGluY3JlYXNlZCB0aGUgYWJ1bmRhbmNlIG9mIGN5YW5vYmFjdGVyaWEgaW4gYWxsIGV4cGVyaW1lbnRzIGFjcm9zcyBhbGwgZml2ZSBsYWtlcyAoIHAgJmFtcDtsdDsgMC4wNSBmb3IgYWxsKSB3aGVyZWFzIGVmZmVjdHMgb24gZXVrYXJ5b3RpYyBwaHl0b3BsYW5rdG9uIHdlcmUgbGltaXRlZC4gU2FuZC1maWx0ZXJlZCBzZXdhZ2UgY29udGFpbmVkIGxlc3MgUCwgb3JnYW5pYyBtYXR0ZXIsIGFuZCBhbW1vbml1bSBidXQgbW9yZSBuaXRyYXRlIGFuZCBoYWQgYSAyNSUgbGVzcyBwb3RlbnQgc3RpbXVsYXRvcnkgZWZmZWN0IG9uIGN5YW5vYmFjdGVyaWEgdGhhbiBtZW1icmFuZS1maWx0ZXJlZCBzZXdhZ2UsIHN1Z2dlc3Rpbmcgbml0cmlmaWNhdGlvbiBtYXkgcGxheSBhIHJvbGUgaW4gcmVkdWNpbmcgQ0hBQnMuIEZlcnRpbGl6ZXIgaW5jcmVhc2VkIG1pY3JvY3lzdGluIGxldmVscyBhbmQgZGVjcmVhc2VkIHRoZSDOtFBPIDE1IE4gd2hlcmVhcyB3YXN0ZXdhdGVyIGluY3JlYXNlZCDOtFBPIDE1IE4gKCBwICZhbXA7bHQ7IDAuMDUgZm9yIGFsbCkuIE1pY3JvY3lzdGlzIHdhcyB0aGUgZ2VudXMgbW9zdCBjb25zaXN0ZW50bHkgcHJvbW90ZWQgYnkgbnV0cmllbnQgc291cmNlcyAoIHAgJmFtcDtsdDsgMC4wNSBpbiBhbGwgZXhwZXJpbWVudHMpLCBmb2xsb3dlZCBieSBDeWFub2JpdW0gKCBwICZhbXA7bHQ7IDAuMDUgaW4gNTAlIG9mIGV4cGVyaW1lbnRzKSwgd2l0aCBpbmNyZWFzZXMgaW4gTWljcm9jeXN0aXMgYmlvbWFzcyBjb25zaXN0ZW50bHkgZWxpY2l0ZWQgYnkgbWVtYnJhbmUtZmlsdGVyZWQgd2FzdGV3YXRlci4gQ29sbGVjdGl2ZWx5LCByZXN1bHRzIGRlbW9uc3RyYXRlIHRoYXQgZGlmZmVyaW5nIHR5cGVzIG9mIHNld2FnZSBkaXNjaGFyZ2UgYW5kIGZlcnRpbGl6ZXJzIGNhbiBwcm9tb3RlIENIQUIgaW50ZW5zaXR5IGFuZCB0b3hpY2l0eSwgd2hpbGUgY29uY3VycmVudGx5IGFsdGVyaW5nIENIQUIgZGl2ZXJzaXR5IGFuZCDOtFBPIDE1IE4uIFdoaWxlIG1lbWJyYW5lLWZpbHRlcmVkIHNld2FnZSBjb25zaXN0ZW50bHkgZmF2b3JlZCBNaWNyb2N5c3RpcyAsIHRoZSBkaXNjaGFyZ2Ugb2Ygc2V3YWdlIHRocm91Z2ggc2FuZHMgbXV0ZWQgYmxvb20gaW50ZW5zaXR5IHN1Z2dlc3Rpbmcgc2FuZC1iZWRzIG1heSByZXByZXNlbnQgYSB0b29sIHRvIHJlbW92ZSBrZXkgbnV0cmllbnRzIGFuZCBwYXJ0aWFsbHkgbWl0aWdhdGUgQ0hBQnMuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjMzODkvZm1pY2IuMjAyNC4xNDY0Njg2IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5mcm9udGllcnNpbi5vcmcvam91cm5hbHMvbWljcm9iaW9sb2d5L2FydGljbGVzLzEwLjMzODkvZm1pY2IuMjAyNC4xNDY0Njg2L3BkZiIsImlzc3VlZERhdGUiOiIyMDI0LTExLTA2IiwiYWNjZXNzZWREYXRlIjoiMjAyNi0wMiIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiRnJvbnRpZXJzIGluIE1pY3JvYmlvbG9neSIsInZvbHVtZSI6MTUsImltcGFjdEZhY3RvciI6IjMuOTkifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjoxNDY0Njg2LCJsYXN0UGFnZSI6MTQ2NDY4Nn19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJGcm9udGllcnMgTWVkaWEifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjczLCJjaXRlZEJ5Q291bnQiOjl9fSwicXVvdGUiOiJkZXJpdmVkIG51dHJpZW50cyBhbHRlciB0aGUgaW50ZW5zaXR5LCBkaXZlcnNpdHksIGFuZCB0b3hpY2l0eSBvZiBoYXJtZnVsIGN5YW5vYmFjdGVyaWFsIGJsb29tcyAuLi4gQ3lhbm9iYWN0ZXJpYWwgaGFybWZ1bCBhbGdhbCBibG9vbXMgKENIQUJzKSBhcmUgcHJvbW90ZWQgYnkgZXhjZXNzaXZlIG51dHJpZW50IGxvYWRpbmcgYW5kLCB3aGlsZSBmZXJ0aWxpemVycyBhbmQgc2V3YWdlIGFyZSB0aGUgbW9zdCBwcmV2YWxlbnQgZXh0ZXJuYWwgbnV0cmllbnQgc291cmNlcyBpbiBtb3N0IHdhdGVyc2hlZHMsIHRoZSBkaWZmZXJlbnRpYWwgZWZmZWN0cyBvZiB0aGVzZSBudXRyaWVudCAuLi4gb3JnYW5pYyBuaXRyb2dlbiAozrRQTzE1Tikgd2VyZSBxdWFudGlmaWVkLiBGZXJ0aWxpemVyIGFuZCBib3RoIHNvdXJjZXMgb2Ygd2FzdGV3YXRlciBpbmNyZWFzZWQgdGhlIGFidW5kYW5jZSBvZiAuLi4gcGh5dG9wbGFua3RvbiB3ZXJlIGxpbWl0ZWQuIFNhbmQtZmlsdGVyZWQgc2V3YWdlIGNvbnRhaW5lZCBsZXNzIFAsIG9yZ2FuaWMgbWF0dGVyLCBhbmQgYW1tb25pdW0gYnV0IG1vcmUgbml0cmF0ZSBhbmQgLi4uIGRpZmZlcmluZyB0eXBlcyBvZiBzZXdhZ2UgZGlzY2hhcmdlIGFuZCBmZXJ0aWxpemVycyBjYW4gcHJvbW90ZSBDSEFCIGludGVuc2l0eSBhbmQgdG94aWNpdHksIHdoaWxlIGNvbmN1cnJlbnRseSAuLi4gc2V3YWdlIHRocm91Z2ggc2FuZHMgbXV0ZWQgYmxvb20gaW50ZW5zaXR5IHN1Z2dlc3Rpbmcgc2FuZC1iZWRzIG1heSByZXByZXNlbnQgYSB0b29sIHRvIHJlbW92ZSBrZXkgbnV0cmllbnRzIGFuZCAuLi4gKCBXYXRzb24gZXQgYWwuLCAyMDE2IDsgQnVsbGVyamFobiBldCBhbC4sIDIwMTYgOyBIdWlzbWFuIGV0IGFsLiwgMjAxOCApIGFuZCBleGNlc3NpdmUgbnV0cmllbnQgbG9hZGluZyBpcyB0aGUgcHJvY2VzcyBtb3N0IGNvbW1vbmx5IGNpdGVkIGFzIHRoZSBlbnZpcm9ubWVudGFsIGZhY3RvciBwcm9tb3RpbmcgQ0hBQnMgKCJ9XQ%3D%3D. Eco-tourism at this Ramsar site waned, losing INR 200 crore amid scum-covered vistas, while invasives like Ipomoea displaced native species (Haxthausen, 2021)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkthcmwgUmFzbXVzIEF1Z3VzdCIsImZhbWlseSI6IkhheHRoYXVzZW4ifX1dLCJpZHMiOnsiaXNzbiI6W10sImlzYm4iOltdLCJtYWciOiIzMTc3MTEzMjA3Iiwib3BlbmFsZXgiOiJXMzE3NzExMzIwNyJ9LCJ0aXRsZSI6IlNlZGltZW50IG1pY3JvYmlhbCBmdWVsIGNlbGxzIGZvciBsYWtlIHJlc3RvcmF0aW9uIiwiYWJzdHJhY3QiOiJFdXRyb3BoaWNhdGlvbiBvZiBsYWtlcyBpcyB1YmlxdWl0b3VzLCBhcyAzMC00MCAlIG9mIHRoZSB3b3JsZOKAmXMgbGFrZXMgYXJlIGV1dHJvcGhpYywgYW5kIGNvbmRpdGlvbnMgY29udGludW91c2x5IGRldGVyaW9yYXRpbmcuIEV1dHJvcGhpY2F0aW9uIGlzIGEgY29uZGl0aW9uIG9mIG92ZXJmZXJ0aWxpemF0aW9uLCB3aGVyZSBhbGwgcmVxdWlyZW1lbnRzIGZvciByYXBpZCBncm93dGggb2YgYWxnYWUgYXJlIG1ldCwgcmVzdWx0aW5nIGluIGFsZ2FsIGJsb29tcy4gVGhpcyBjYXVzZXMgYSBzZWxmLXN1c3RhaW5lZCwgdmljaW91cyBjeWNsZSB3aGVyZSBwb29yIGVjb2xvZ2ljYWwgY29uZGl0aW9ucyBhcmUgbWFpbnRhaW5lZCB1bnRpbCBudXRyaWVudHMgYXJlIGxvc3QgZnJvbSB0aGUgbGFrZSBvdmVyIG1hbnkgeWVhcnMuIExha2UgcmVzdG9yYXRpb24gc2Vla3MgdG8gYnJlYWsgdGhpcyB2aWNpb3VzIGN5Y2xlLiBJbmFkZXF1YXRlIGF2YWlsYWJpbGl0eSBvZiBhbnkgc2luZ2xlIG51dHJpZW50IGluIHdpbGwgbGltaXQgYWxnYWwgZ3Jvd3RoLCB0eXBpY2FsbHkgcGhvc3Bob3J1cyBpcyB0YXJnZXRlZC4gVGhlIHNjb3BlIG9mIHRoaXMgUGhEIHRoZXNpcyB3YXMgdG8gZGV2ZWxvcCwgdGVzdCBhbmQgb3B0aW1pemUgc2VkaW1lbnQgbWljcm9iaWFsIGZ1ZWwgY2VsbHMgKFNNRkMpIGFzIGEgbm92ZWwgbWV0aG9kIGZvciBsYWtlIHJlc3RvcmF0aW9uLiBUaGUgdGhlc2lzIHNwYW5zIHRoZSB0d28gbWFpbiB0b3BpY3Mgb2YgU01GQ3MgYW5kIGxha2UgcmVzdG9yYXRpb24gdGVjaG5pcXVlcy4gTGFrZSByZXN0b3JhdGlvbiB0ZWNobmlxdWVzIGFyZSByZXZpZXdlZCwgcHJvdmlkaW5nIHVuZGVyc3RhbmRpbmcgb2YgdGhlIHVuZGVybHlpbmcgbWVjaGFuaXNtcy4gU01GQ3MgYXJlIGFwcHJvYWNoZWQgZnJvbSBhbiBlbmdpbmVlcmluZyBwZXJzcGVjdGl2ZSBhbmQgYSBmb2N1cyBvbiBsYWtlIHJlbWVkaWF0aW9uLiBUaGUgc3BlY2lmaWMgb2JqZWN0aXZlcyB3ZXJlIHRvIHRlc3QgYSBsb3ctY29zdCBlbGVjdHJvZGUgbWF0ZXJpYWwgZm9yIGl0cyBkdXJhYmlsaXR5LCBwcmFjdGljYWxpdHksIGFuZCBwZXJmb3JtYW5jZS4gUGVyZm9ybWFuY2Ugd2FzIGFzc2Vzc2VkIGFzIHRoZSBhYmlsaXR5IHRvIGxpbWl0IHBob3NwaG9ydXMgcmVsZWFzZSBmcm9tIHNlZGltZW50cywgaW5oaWJpdCBtZXRoYW5lIHJlbGVhc2UsIGFuZCB0aGUgYWRkZWQgYmVuZWZpdHMgb2YgZGVncmFkaW5nIHBvbGx1dGFudHMsIGFuZCBnZW5lcmF0aW5nIGVsZWN0cmljaXR5LiBQcm9vZiBvZiBjb25jZXB0IHdhcyBzaG93biBpbiBsYWJvcmF0b3J5IHJlYWN0b3JzICgxIEwpLCB0aGUgZWZmZWN0IG9mIHVwc2NhbGluZyB0byAyMDAgTCBpbiBuYXR1cmFsIGVudmlyb25tZW50IHdhcyBzdHVkaWVkIGluIG91dGRvb3IgbWVzb2Nvc21zLCBhbmQgdGhlbiB0ZXN0ZWQgYXQgcGlsb3Qgc2NhbGUgaW4gYSBldXRyb3BoaWMgZnJlc2h3YXRlciBsYWtlLiBQaWxvdCBpbnN0YWxsYXRpb25zIHdlcmUgdGhlIGxhcmdlc3QgaW4gdGhlIHdvcmxkIG9mIHRoZWlyIGtpbmQgYW5kIHdlcmUgb3BlcmF0ZWQgZm9yIDIgeWVhcnMuIFN0YWlubGVzcyBzdGVlbCB3YXMgc2VsZWN0ZWQgZm9yIHRlc3RpbmcgYXMgZWxlY3Ryb2RlIG1hdGVyaWFsIGR1ZSB0byBpdHMgbG93IGNvc3QsIGR1cmFiaWxpdHksIGFuZCBhdmFpbGFiaWxpdHkuIFBhaXJzIG9mIGVsZWN0cm9kZXMgd2VyZSBlbWJlZGRlZCBpbiB0aGUgc2VkaW1lbnQgKHRoaXMgaXMgY2FsbGVkIHRoZSBhbm9kZSkgYW5kIHN1c3BlbmRlZCBpbiB0aGUgb3Zlcmx5aW5nIHdhdGVyICh0aGlzIGlzIGNhbGxlZCB0aGUgY2F0aG9kZSkuIFRoZSB0d28gZWxlY3Ryb2RlcyB3ZXJlIGNvbm5lY3RlZCBhY3Jvc3MgYW4gZXh0ZXJuYWwgcmVzaXN0b3Igd2l0aCBlbGVjdHJpY2FsIHdpcmVzLiBNaWNyb2JlcyBpbiB0aGUgc2VkaW1lbnQgdXNlIHRoZSBhbm9kZSBwYWlyIGFzIGVsZWN0cm9uIGFjY2VwdG9yLCBlbmFibGluZyByZXNwaXJhdGlvbiBpbiB0aGUgYW5veGljIHNlZGltZW50LiBFbGVjdHJvbnMgdHJhdmVsIGZyb20gdGhlIHNlZGltZW50IHZpYSB0aGUgZWxlY3RyaWNhbCBjb25uZWN0b3IgdG8gdGhlIG92ZXJseWluZyBjYXRob2RlIHdoZXJlIG94eWdlbiBpbiB0aGUgd2F0ZXIgaXMgdGhlIHRlcm1pbmFsIGVsZWN0cm9uIGFjY2VwdG9yIGNvbXBsZXRpbmcgdGhlIGNpcmN1aXQuIEFzIGFuIGFkZGVkIGJlbmVmaXQsIGRlZ3JhZGF0aW9uIG9mIHBvbGx1dGFudHMgdXNpbmcgU01GQyB3YXMgdGVzdGVkIGluIG1hcmluZSBoYXJib3VyIHNlZGltZW50cywgYnV0IG5vIGNvbmNsdXNpdmUgcmVzdWx0cyBvYnRhaW5lZC4gTGFib3JhdG9yeSByZWFjdG9ycyB3ZXJlIG9wZXJhdGVkIGZvciBtb3JlIHRoYW4gMjAwIGRheXMgdXNpbmcgcGV0cm9sZXVtIGFzIHNvbGUgY2FyYm9uIHNvdXJjZSwgY29uZmlybWluZyBvaWwgZGVncmFkYXRpb24gYnkgU01GQ3MuIEEgcmVtb3ZhbCByYXRlIG9mIDAuOSBnIHBldHJvbGV1bSBtLTMgZC0xIHJlYWN0b3Igdm9sdW1lIHdhcyBlc3RpbWF0ZWQgZnJvbSBtZWFzdXJlZCBlbGVjdHJpY2FsIG91dHB1dC4gVGhpcyB3YXMgZG9uZSBieSBhc3N1bXB0aW9uIG9mIHRoZSBjaGVtaWNhbCByZWFjdGlvbiBvY2N1cnJpbmcsIGZyb20gdGhpcyB0aGUgbnVtYmVyIG9mIGVsZWN0cm9ucyB0cmFuc2ZlcnJlZCBmcm9tIG9pbCB0byB3YXRlciB3YXMga25vd24gYW5kIGNvdWxkIGJlIGNvcnJlbGF0ZWQgdG8gdGhlIG1lYXN1cmVkIGZsb3cgb2YgZWxlY3Ryb25zIChjdXJyZW50KSB0aHJvdWdoIHRoZSBleHRlcm5hbCBjaXJjdWl0LiBTdGFpbmxlc3Mgc3RlZWwgZWxlY3Ryb2RlcyB3ZXJlIG9wZXJhdGVkIGluIGZyZXNod2F0ZXIgbGFrZSBlbnZpcm9ubWVudHMgZm9yIG1vcmUgdGhhbiA0IHllYXJzIHdpdGggbWluaW1hbCBkYW1hZ2UgdG8gdGhlIG1hdGVyaWFsLiBUaGUgZWxlY3Ryb2RlIGlzIHRoZSBtYWluIG1hdGVyaWFsIGNvc3QgaW4gc3VjaCBzeXN0ZW1zLiBIb3dldmVyLCBBSVNJMzE2IHN0YWlubGVzcyBzdGVlbCBzaW1pbGFyIHRvIHdoYXQgd2FzIHVzZWQgaW4gdGhpcyB0aGVzaXMgd2FzIGF0IHRoZSB0aW1lIG9mIHdyaXRpbmcgYXZhaWxhYmxlIGF0IGJ1bGsgY29zdCBvZiAwLjA5MSDigqwgbS0yLCBjb3JyZXNwb25kaW5nIHRvIDkxMCDigqwgYWNyZS0xLCBzaWduaWZpY2FudGx5IGxvd2VyIHRoYW4gb3RoZXIgY29tbW9uIG1hdGVyaWFscy4gQ2F0aG9kZXMgYXQgbGFyZ2Ugc2NhbGUgY291bGQgYmUgbW92ZWQgaG9yaXpvbnRhbGx5IGFuZCB2ZXJ0aWNhbGx5IHdpdGggbWlub3IgaW1wYWN0IHRvIHBlcmZvcm1hbmNlLiBQaWxvdCBzY2FsZSBTTUZDcyBnZW5lcmF0ZWQgMC4zODMgbVcgbS0yIHN1c3RhaW5lZCBhbmQgMjcuNCBtVyBtLTIgcGVhayBvdXRwdXQuIFRoZSBzeXN0ZW0gd2FzIGxpbWl0ZWQgYnkgdGhlIGNhdGhvZGUgc3VyZmFjZSBhcmVhLCBtZWFuaW5nIG11bHRpLWxheWVyIGNhdGhvZGVzIHNob3VsZCBiZSB0ZXN0ZWQgaW4gdGhlIGZ1dHVyZSBpZiBlbGVjdHJpY2FsIHBvd2VyIHByb2R1Y3Rpb24gaXMgdGhlIGdvYWwuIFRoZSBjYXRob2RlIHNob3VsZCBiZSBwbGFjZWQgYXQgYSBkZXB0aCB0aGF0IGVuc3VyZXMgaGlnaCBkaXNzb2x2ZWQgb3h5Z2VuIGNvbmNlbnRyYXRpb25zIHRvIGF2b2lkIHRoZSBzeXN0ZW0gdG8gYmUgbGltaXRlZC4gSG93ZXZlciwgdG9vIGhpZ2ggZWxlY3RyaWNpdHkgZ2VuZXJhdGlvbiB3YXMgZm91bmQgY291bnRlcnByb2R1Y3RpdmUgZm9yIHJlbWVkaWF0aW9uIHB1cnBvc2VzIGJlY2F1c2UgdGhpcyB3b3VsZCBpbmNyZWFzZSBzZWRpbWVudCB0dXJub3ZlciByYXRlLCByZWxlYXNpbmcgcG9sbHV0YW50cyBzdG9yZWQgaW4gdGhlIHNlZGltZW50cy4gUCBiaW5kaW5nIHdhcyBub3QgZm91bmQgdG8gcmVxdWlyZSBoaWdoIHR1cm5vdmVyIG9mIHNlZGltZW50IG9yZ2FuaWMgbWF0dGVyLiBUaGUgdHVybm92ZXIgY2FuIGJlIGNvbnRyb2xsZWQgYnkgY2hhbmdpbmcgdGhlIGV4dGVybmFsIHJlc2lzdGFuY2UuIEhpZ2ggcmVzaXN0YW5jZSBsaW1pdHMgdGhlIGVsZWN0cmljYWwgcGVyZm9ybWFuY2UsIGxvdyByZXNpc3RhbmNlIGluY3JlYXNlcyBzZWRpbWVudCB0dXJub3ZlciBhbmQgcmlza3MgZGVzdGFiaWxpemluZyB0aGUgc3lzdGVtLiBUbyBhY2hpZXZlIHJlZHVjZWQgY29uY2VudHJhdGlvbnMgb2YgcGhvc3BoYXRlIGluIHRoZSB3YXRlciBjb2x1bW4sIGFub2RlcyBtdXN0IGJlIHBsYWNlZCBpbiB0aGUgc2VkaW1lbnQgc3VyZmFjZSBzaW5jZSBhIGZldyBtaWxsaW1ldHJlcyBvZiBzZWRpbWVudCBtYXkgY29udGFpbiBhbmQgcmVsZWFzZSBlbm91Z2ggUCB0byBzdXN0YWluIGV1dHJvcGhpY2F0aW9uLiBQaG9zcGhhdGUgd2FzIG1lYXN1cmVkIGluIHRoZSB3YXRlciBjb2x1bW4gbmVhciB0aGUgc2VkaW1lbnQsIGluIHN5c3RlbXMgd2l0aCBhbmQgd2l0aG91dCBlbGVjdHJvZGVzLiBQaG9zcGhhdGUgY29uY2VudHJhdGlvbnMgd2VyZSBzdWNjZXNzZnVsbHkgcmVkdWNlZCBieSB1cCB0byA5NSAlIGluIDItY2hhbWJlciBsYWJvcmF0b3J5IHJlYWN0b3JzLCBhbmQgYnkgdXAgdG8gOTggJSBpbiB0aGUgcGlsb3QgaW5zdGFsbGF0aW9uLiBJbiB0aGUgcGlsb3QgZXhwZXJpbWVudCwgcGhvc3BoYXRlIGNvbmNlbnRyYXRpb25zIGFib3ZlIGVsZWN0cm9kZXMgd2VyZSBsb3dlciBpbiBhbGwgbWVhc3VyZW1lbnRzLCBjb21wYXJlZCB0byBjb250cm9scy4gU2VkaW1lbnQgcmVkb3ggcG90ZW50aWFscyBpbmNyZWFzZWQgZnJvbSAtNDAwIG1WIHZzIHN0YW5kYXJkIGh5ZHJvZ2VuIGVsZWN0cm9kZSAoU0hFKSB0byAtMTc1IG1WIHZzIFNIRSwgY29ycmVzcG9uZGluZyB0byBhIG1vdmUgZnJvbSBhIG1ldGhhbm9nZW5ldGljIGRvbWFpbiB0byBzdWxwaHVyIHJlZHVjaW5nLiBBbmFseXNpcyBvZiB0aGUgbWljcm9iaWFsIGNvbW11bml0eSBpbmRpY2F0ZWQgZW5oYW5jZWQgc3VscGh1ciBjeWNsaW5nIHdoZXJlIGVsZWN0cm9kZXMgb3hpZGl6ZSBzdWxwaGlkZSB3aGljaCB0aGUgYmFjdGVyaWEgYWdhaW4gcmVkdWNlIHRvIG9idGFpbiBlbmVyZ3kuIEl0IHdhcyBoeXBvdGhlc2lzZWQgdGhhdCBTTUZDIHByb21vdGUgaGlnaGVyIHJlZG94IHBvdGVudGlhbHMgYnkgZmFjaWxpdGF0aW5nIG94aWRhdGlvbiBvZiBzdWxwaHVyIGFuZCBpcm9uIHNwZWNpZXMsIGluaGliaXRpbmcgSDJTIGZvcm1hdGlvbiBhbmQgRmUgc2NhdmVuZ2luZyBieSBGZVMgZm9ybWF0aW9uLCB0aHVzIGFsbG93aW5nIGZvciBQIGJpbmRpbmcuIEEgbG93ZXIgcmVsYXRpdmUgYWJ1bmRhbmNlIG9mIG1ldGhhbm9nZW5zIG9uIGVsZWN0cm9kZXMgY29tcGFyZWQgdG8gY29udHJvbHMgc3VnZ2VzdHMgc3VwcHJlc3Npb24gb3Igb3V0Y29tcGV0aW5nIG9mIG1ldGhhbm9nZW5zLlF1YW50aWZpY2F0aW9uIG9mIG1ldGhhbmUgcHJvZHVjdGlvbiB3YXMgZG9uZSBvbiB0aGUgYmFzaXMgb2YgYSBDSDQ6Q08yIHJhdGlvIHRvIGFsbG93IGNvbXBhcmFiaWxpdHkgYmV0d2VlbiBzeXN0ZW1zLiBNZXRoYW5lIGVtaXNzaW9uIHdhcyByZWR1Y2VkIGJ5IHVwIHRvIDk5ICUgYW5kIG1haW50YWluZWQgaW5oaWJpdGlvbiBmb3Igc2V2ZXJhbCBtb250aHMgaW4gbGFib3JhdG9yeSBzeXN0ZW1zIHdpdGggZWxlY3Ryb2Rlcy4gSW4gdGhlIHBpbG90IGluc3RhbGxhdGlvbnMgbm8gZ2FzIHNhbXBsZXMgY291bGQgYmUgY29sbGVjdGVkLCBhbmQgbm8gQ08yOkNINCByYXRpb3Mgd2VyZSBvYnRhaW5lZC4gQWVyYXRpb24gaXMgdGhlIGN1cnJlbnQgcmVzdG9yYXRpb24gdGVjaG5pcXVlIHRoYXQgbW9zdCBjbG9zZWx5IHJlc2VtYmxlcyB0aGUgcHJvcG9zZWQgU01GQyB0ZWNobmlxdWUuIEFlcmF0aW9uIGhhcyBwcmV2aW91c2x5IGZhaWxlZCB0byByZWR1Y2UgUCByZWxlYXNlIGluIHNldmVyYWwgc2VwYXJhdGUgaW52ZXN0aWdhdGlvbnMuIFNNRkNzIGFzIGEgcmVtZWRpYXRpb24gdG9vbCB3YXMgZm91bmQgdG8gb2ZmZXIgc2V2ZXJhbCBiZW5lZml0cyBvdmVyIGNvbnZlbnRpb25hbCBtZXRob2RzOiBTTUZDcyBvZmZlcnMgY29udGludW91cyBlZmZlY3QsIHdoaWxlIG90aGVyIHRlY2huaXF1ZXMgcmVxdWlyZSBjb250aW51b3VzIG9yIHJlcGVhdGVkIHRyZWF0bWVudC4gU01GQ3MgY2FuIGJlIGNvbnRyb2xsZWQgaW4gcmVhbC10aW1lIGJ5IGFsdGVyaW5nIHRoZSBleHRlcm5hbCByZXNpc3RvciBhbmQgY2FuIGJlIGVhc2lseSBtb25pdG9yZWQgYnkgbWVhc3VyaW5nIHRoZSBwcm9kdWNlZCBlbGVjdHJpY2FsIG91dHB1dC4gSWYgaW1wbGVtZW50ZWQgYXQgbGFyZ2Ugc2NhbGUgaXQgaXMgZXhwZWN0ZWQgdGhhdCB0aGUgZWxlY3Ryb2RlIGZpZWxkcyB3aWxsIGhlbHAgbWFpbnRhaW4gbG93IHBob3NwaGF0ZSBjb25jZW50cmF0aW9ucyBpbiB0aGUgd2F0ZXIgZm9yIGRlY2FkZXMgd2l0aCBtaW5pbWFsIGludGVydmVudGlvbiwgYW5kIGFmdGVyIHNvbWUgdGltZSwgYmVudGhpYyBwbGFudHMgYXJlIGV4cGVjdGVkIHRvIHJldHVybiwgdGh1cyBtYWludGFpbmluZyB0aGUgYmFsYW5jZSBoZW5jZSBmb3J0aC4iLCJ1cmwiOiJodHRwczovL2xvY2FsLmZvcnNrbmluZ3Nwb3J0YWwuZGsvbG9jYWwvZGtpLWNnaS93cy9jcmlzLWxpbms%2Fc3JjPWR0dSZpZD1kdHUtZDY2ZDMzNzAtMjU2Zi00ODg1LWEwOWUtMzM2YTkyNGY0YmVkJnRpPVNlZGltZW50JTIwbWljcm9iaWFsJTIwZnVlbCUyMGNlbGxzJTIwZm9yJTIwbGFrZSUyMHJlc3RvcmF0aW9uIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2JhY2tlbmQub3JiaXQuZHR1LmRrL3dzL2ZpbGVzLzI0ODA2OTU4OS9UaGVzaXNfb25saW5lX3ZlcnNpb25fS2FybF9IYXh0aGF1c2VuLnBkZiIsImlzc3VlZERhdGUiOiIyMDIxLTAxLTAxIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0wOCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiUmVzZWFyY2ggUG9ydGFsIERlbm1hcmsiLCJpbXBhY3RGYWN0b3IiOiIwLjA0In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6NzZ9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiVGVjaG5pY2FsIFVuaXZlcnNpdHkgb2YgRGVubWFyayJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MCwiY2l0ZWRCeUNvdW50IjoxfX0sInF1b3RlIjoiRXh0ZXJuYWwgaW5wdXRzIGluY3JlYXNlIFAgY29uY2VudHJhdGlvbiBpbiAgdGhlIHdhdGVyLCBjYXVzaW5nIGFsZ2FsIGJsb29tcyB0aGF0IHNoYWRvdyB0aGUgbGFrZSBib3R0b20sIGFuZCBiZW50aGljIG1hYy1yb3BoeXRlcyByZWNlZGUuIFByZWRhdG9yeSBmaXNoIGFyZSB1bmFibGUgdG8gaHVudCwgYW5kIHRoZWlyIGZvb2Qgc291cmNlIGlzICBkaXNydXB0ZWQgc3VjaCBhcyBmaXNoIGZlZWRpbmcgb24gYmVudGhpYyBpbnZlcnRlYnJhdGVzIGFuZCBib3R0b20tZHdlbGxpbmcgYW5pbWFscyBhcmUgc3VmZm9jYXRlZCAoVmFuZGVyIFphbmRlbiBhbmQgVmFkZWJvbmNvZXVyIDIwMDIpLCBzbyB0aGF0IHBsYW5rdGl2b3JvdXMgZmlzaCBzcGVjaWVzIGRvbWluYXRlLiBUaGlzIGluY3JlYXNlcyBwcmVkYXRpb24gb24gem9vIHBsYW5rdG9uICBhbmQgcmVkdWNlcyBncmF6aW5nIG9uIGFsZ2FlLiBBbGdhZSBwcm9saWZlcmF0ZSwgZGllLCBhbmQgc2luayB0byB0aGUgYm90dG9tLCAgY29uc3VtaW5nIG94eWdlbiB3aGVuIGRlZ3JhZGVkLCB3aGljaCBsZWFkcyB0byBveHlnZW4gZGVmaWNpdCBpbiB0aGUgYm90LXRvbSwga2lsbGluZyBmaXNoIChXdXJ0c2JhdWdoIGV0IGFsLiAyMDE5KS4gRmlndXJlIDI6IFZpY2lvdXMgY3ljbGVzIG9mIGV1dHJvcGhpY2F0aW9uLiBMZWZ0OiBJbnRlcmxpbmtlZCBwcm9jZXNzZXMgc3VzdGFpbmluZyBldXRyb3BoaS1jYXRpb24gaW4gYSBsYWtlLiBCbHVlOiBwcm9jZXNzZXMgaW4gd2F0ZXIsIGdyZWVuOiBwcm9jZXNzZXMgaW52b2x2aW5nIG1pY3JvYWxnYWUsIGdyZXk6IHNlZC1pbWVudCByZWxhdGVkIHByb2Nlc3NlcywgeWVsbG93OiBwcm9jZXNzZXMgaW52b2x2aW5nIGZhdW5hLiBSaWdodCwgZGFzaGVkIGxpbmU6IFRoZSBvcmRlciBpbiB3aGljaCBlbGVjdHJvbiBhY2NlcHRvcnMgYXJlIGNvbnN1bWVkIGluIHRoZSBhYnNlbmNlIG9mIE8yIHNob3duIGFzIGNvbmNlbnRyYXRpb24gdmVyc3VzICBkZXB0aCBpbiBzZWRpbWVudCwgYW5kIGF0IHdoaWNoIHBvdGVudGlhbCB0aGlzIG9jY3VycyAoQmVybmVyIDE5ODEsIGJ5IGNvdXJ0ZXN5IG9mIHRoZSBzb2NpZXR5ICBmb3Igc2VkaW1lbnRhcnkgZ2VvbG9neSkuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxNywiYmJveCI6eyJ0b3AiOjc5LjkzMzE1MTI0NTExNzE5LCJsZWZ0IjoxNC4yOTU1MDM2MTYzMzMwMDgsIndpZHRoIjo3MS45Mzg5MzQzMjYxNzE4OCwiaGVpZ2h0IjoyLjA1MzQ3NzI4NzI5MjQ4MDV9fSx7InBhZ2VOdW1iZXIiOjE3LCJiYm94Ijp7InRvcCI6ODEuOTcxMzQzOTk0MTQwNjIsImxlZnQiOjE0LjI5NTUwMzYxNjMzMzAwOCwid2lkdGgiOjcxLjMzMzAwMDE4MzEwNTQ3LCJoZWlnaHQiOjIuMDUzNDc3Mjg3MjkyNDgwNX19LHsicGFnZU51bWJlciI6MTcsImJib3giOnsidG9wIjo4NC4wMjM3OTYwODE1NDI5NywibGVmdCI6MTQuMjk1NTAzNjE2MzMzMDA4LCJ3aWR0aCI6NzEuODEyMDE5MzQ4MTQ0NTMsImhlaWdodCI6Mi4wNTM0NzcyODcyOTI0ODA1fX0seyJwYWdlTnVtYmVyIjoxNywiYmJveCI6eyJ0b3AiOjg2LjA2MTk5NjQ1OTk2MDk0LCJsZWZ0IjoxNC4yOTU1MDM2MTYzMzMwMDgsIndpZHRoIjo3MS45NTczNTkzMTM5NjQ4NCwiaGVpZ2h0IjoyLjA1MzQ3NzI4NzI5MjQ4MDV9fSx7InBhZ2VOdW1iZXIiOjE4LCJiYm94Ijp7InRvcCI6OS45MjY0ODc5MjI2Njg0NTcsImxlZnQiOjE0LjI5NTUwMzYxNjMzMzAwOCwid2lkdGgiOjcxLjk3NDA2MDA1ODU5Mzc1LCJoZWlnaHQiOjEwLjIzNDc3NzQ1MDU2MTUyM319LHsicGFnZU51bWJlciI6MTgsImJib3giOnsidG9wIjo0NC43MzQwNDMxMjEzMzc4OSwibGVmdCI6MTQuMjk1NTAzNjE2MzMzMDA4LCJ3aWR0aCI6NzEuODkzNDMyNjE3MTg3NSwiaGVpZ2h0IjoxMC40MDMyODk3OTQ5MjE4NzV9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI5MyJ9fV0%3D. Adaptive inlet management revived hydrology temporarily, but nutrient legacies and cyclones project 2-3 fold bloom intensification by 2050 (Jansen et al., 2024; Meerhoff et al., 2022)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik0uIiwiZmFtaWx5IjoiTWVlcmhvZmYifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSi4iLCJmYW1pbHkiOiJBdWRldCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJULkEuIiwiZmFtaWx5IjoiRGF2aWRzb24ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTC4gRGUiLCJmYW1pbHkiOiJNZWVzdGVyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlMuIiwiZmFtaWx5IjoiSGlsdCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTLiIsImZhbWlseSI6Iktvc3RlbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJaYWlodWEiLCJmYW1pbHkiOiJMaXUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTi4iLCJmYW1pbHkiOiJNYXp6ZW8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSGFucyBXLiIsImZhbWlseSI6IlBhZXJsIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik0uIiwiZmFtaWx5IjoiU2NoZWZmZXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiRS4iLCJmYW1pbHkiOiJKZXBwZXNlbiJ9fV0sImlkcyI6eyJpc3NuIjpbXSwiaXNibiI6W10sImRvaSI6IjEwLjE3NjE1Lzd2ZTUtdzAxNiIsIm9wZW5hbGV4IjoiVzQzMTg0MTU1NzcifSwidGl0bGUiOiJGZWVkYmFjayBiZXR3ZWVuIGNsaW1hdGUgY2hhbmdlIGFuZCBldXRyb3BoaWNhdGlvbjogcmV2aXNpdGluZyB0aGUgYWxsaWVkIGF0dGFjayBjb25jZXB0IGFuZCBob3cgdG8gc3RyaWtlIGJhY2siLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTc2MTUvN3ZlNS13MDE2IiwicmVzb3VyY2VVcmwiOiJodHRwczovL2Nkci5saWIudW5jLmVkdS9kb3dubG9hZHMvaDk4OXJjOTM4IiwiaXNzdWVkRGF0ZSI6IjIwMjItMDEtMDEiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTEwIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJDYXJvbGluYSBEaWdpdGFsIFJlcG9zaXRvcnkgKFVuaXZlcnNpdHkgb2YgTm9ydGggQ2Fyb2xpbmEgYXQgQ2hhcGVsIEhpbGwpIiwiaW1wYWN0RmFjdG9yIjoiMC4xMCJ9LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJVbml2ZXJzaXR5IG9mIE5vcnRoIENhcm9saW5hIGF0IENoYXBlbCBIaWxsIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjowLCJjaXRlZEJ5Q291bnQiOjN9fSwicXVvdGUiOiJXaXRoIGNsaW1hdGUgd2FybWluZywgdGhlIGRpcmVjdCBhbmQgaW5kaS1yZWN0IGVmZmVjdHMgb2YgaW5jcmVhc2VkIGxha2UgdGVtcGVyYXR1cmVzIHdpbGwgbGlrZWx5IGRyaXZlIGNoYW5nZXMgaW4gY3lhbm9iYWN0ZXJpYWwgdGF4YSBhbmQgdGhlaXIgcHJvZHVjLXRpb24gb2YgY3lhbm9iYWN0ZXJpYWwgdG94aW5zLCBsaWtlbHkgaW5jbHVkaW5nIHByb21vdC1pbmcgdGhlIHNlbGVjdGlvbiBvZiBhIGZldyBNaWNyb2N5c3RpcyBzcGVjaWVzIG9yIHN0cmFpbnMgcHJvZHVjaW5nIGhpZ2hseSB0b3hpYyBtaWNyb2N5c3Rpbi1MUiAoTWFudHpvdWtpIGV0IGFsLiAyMDE4KS4gRGVzcGl0ZSB2YXJpYXRpb25zIGFtb25nIGxha2VzIGluIGEgd2F0ZXItc2NhcGUsIGEgcmVnaW9uYWwgaW5jcmVhc2UgaW4gbWljcm9jeXN0aW4gbWF4aW1hIGFuZCBkdXJhdGlvbiBvZiBibG9vbXMgaGFzIGFsc28gYmVlbiBjbGVhcmx5IGV4cGxhaW5lZCBieSBpbmNyZWFzaW5nIHRlbXBlcmF0dXJlIGluIGNlbnRyYWwgTm9ydGggQW1lcmljYSAoSGF5ZXMgZXQgYWwuIDIwMjApLCB3aXRoIGluY3JlYXNpbmcgcmlza3MgdG8gaHVtYW4gaGVhbHRoLiBHcm93aW5nIGV2aWRlbmNlIGluZGljYXRlcyB0aGF0IGN5YW5vYmFjdGVyaWFsIGRvbWluYW5jZSB3aWxsIGV4ZXJ0IGEgcG9zaXRpdmUgZmVlZGJhY2sgd2l0aCBjbGltYXRlIHdhcm1pbmcgKEhhdmVucyBldCBhbC4gMjAxNiwgUGFlcmwgZXQgYWwuIDIwMjApLiBNYW55IGN5YW5vYmFjdGVyaWFsIHNwZWNpZXMgcHJvbGlmZXJhdGUgb24gdGhlIGxha2Ugc3VyZmFjZSwgYWJzb3JiaW5nIGxpZ2h0LCBlbmhhbmNpbmcgbG9jYWxpemVkIHRlbXBlcmF0dXJlIGluY3JlYXNlcywgYW5kIGluY3JlYXNpbmcgdGhlcm1hbCBzdHJhdGlmaWNhdGlvbiBwb3RlbnRpYWwgKEthaHJ1IGV0IGFsLiAxOTkzKS4gQ3lhbm9iYWN0ZXJpYSBwcm9tb3RlIHRoZSByZWxlYXNlIG9mIEdIR3MsIHBhcnRpY3VsYXJseSBDSDQgKFlhbiBldCBhbC4gMjAxNyksIGR1cmluZyB0aGUgZGVjb21wb3NpdGlvbiBmb2xsb3dpbmcgdGhlIGNvbGxhcHNlIG9mIGEgYmxvb20uIEN5YW5vYmFjdGVyaWEgYWxzbyBwcm9tb3RlIHJlbWluZXJhbGl6YXRpb24gb2YgbnV0cmktZW50cyBkdXJpbmcgYmxvb21zLCBsZWFkaW5nIHRvIG11bHRpcGxlIGJsb29tcyBpbiBhIHNpbmdsZSBzZWFzb24uIFVuZGVyIGV1dHJvcGhpYyBhbmQgd2FybWluZyBjb25kaS10aW9ucywgY3lhbm9iYWN0ZXJpYSBkb21pbmFuY2UgbWF5IHRodXMgYmUgYSBzZWxmLXBlcnBldHVhdGluZyBwcm9waGVjeSAoRmlnLiAyIGFuZCAzKS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo2Ny4wNjM0MDAyNjg1NTQ2OSwibGVmdCI6NTEuNDc4ODAxNzI3Mjk0OTIsIndpZHRoIjozOC42NzM0MjM3NjcwODk4NDQsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6NjguNjk4NTQ3MzYzMjgxMjUsImxlZnQiOjUxLjQ3ODgwMTcyNzI5NDkyLCJ3aWR0aCI6MzguNjc5NDk2NzY1MTM2NzIsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6NzAuMzMzNjg2ODI4NjEzMjgsImxlZnQiOjUxLjQ3ODgwMTcyNzI5NDkyLCJ3aWR0aCI6MzguNjc3NTgxNzg3MTA5Mzc1LCJoZWlnaHQiOjEuNDcxMTAxNTIyNDQ1Njc4N319LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjcxLjk2ODgyNjI5Mzk0NTMxLCJsZWZ0Ijo1MS40Nzg4MDE3MjcyOTQ5Miwid2lkdGgiOjM4LjY3OTEwMzg1MTMxODM2LCJoZWlnaHQiOjEuNDcxMTAxNTIyNDQ1Njc4N319LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjczLjYxMDU3MjgxNDk0MTQsImxlZnQiOjUxLjQ3ODgwMTcyNzI5NDkyLCJ3aWR0aCI6MzguNjc0NDI3MDMyNDcwNywiaGVpZ2h0IjoxLjQ3Mzc0Njg5NTc5MDF9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo3NS4yNDcwMzk3OTQ5MjE4OCwibGVmdCI6NTEuNDc4ODA5MzU2Njg5NDUsIndpZHRoIjozOC42NzQyNjMwMDA0ODgyOCwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo3Ni44ODIxODY4ODk2NDg0NCwibGVmdCI6NTEuNDc4ODA5MzU2Njg5NDUsIndpZHRoIjozOC42ODExNzE0MTcyMzYzMywiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo3OC41MjUyNTMyOTU4OTg0NCwibGVmdCI6NTEuNDc4ODA5MzU2Njg5NDUsIndpZHRoIjozOC42NzU2NjI5OTQzODQ3NjYsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODAuMTYwNDAwMzkwNjI1LCJsZWZ0Ijo1MS40Nzg4MDkzNTY2ODk0NSwid2lkdGgiOjM4LjY4MjAyOTcyNDEyMTA5NCwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo4MS43OTU1Mzk4NTU5NTcwMywibGVmdCI6NTEuNDc4ODA5MzU2Njg5NDUsIndpZHRoIjozOC42NzQ5ODM5NzgyNzE0ODQsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODMuNDM4NjEzODkxNjAxNTYsImxlZnQiOjUxLjQ3ODgwOTM1NjY4OTQ1LCJ3aWR0aCI6MzguNjc2MTg1NjA3OTEwMTU2LCJoZWlnaHQiOjEuNDg2OTczNTI0MDkzNjI4fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODUuMDg5NjMwMTI2OTUzMTIsImxlZnQiOjUxLjQ3ODgwOTM1NjY4OTQ1LCJ3aWR0aCI6NC42ODI4MDUwNjEzNDAzMzIsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODYuNzI0Nzc3MjIxNjc5NjksImxlZnQiOjUzLjQ0MTE3NzM2ODE2NDA2LCJ3aWR0aCI6Ni4xNDAzNTU1ODcwMDU2MTUsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODYuNzI0ODkxNjYyNTk3NjYsImxlZnQiOjYwLjk5NDg3Njg2MTU3MjI2Niwid2lkdGgiOjYuMDM2OTkyNTQ5ODk2MjQsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODYuNzI0ODkxNjYyNTk3NjYsImxlZnQiOjY4LjQ0NjUyNTU3MzczMDQ3LCJ3aWR0aCI6Ni4xNTA3MDU4MTQzNjE1NzIsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODYuNzI0ODkxNjYyNTk3NjYsImxlZnQiOjc2LjAwOTgyNjY2MDE1NjI1LCJ3aWR0aCI6Mi43MjczMzMzMDcyNjYyMzU0LCJoZWlnaHQiOjEuNDcxMTAxNTIyNDQ1Njc4N319LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjg2LjcyNDg5MTY2MjU5NzY2LCJsZWZ0Ijo4MC4xNDk1ODE5MDkxNzk2OSwid2lkdGgiOjEwLjAwODIzNDk3NzcyMjE2OCwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo4OC4zNjAwMzg3NTczMjQyMiwibGVmdCI6NTEuNDc3NjU3MzE4MTE1MjM0LCJ3aWR0aCI6MzguNjcyMDU0MjkwNzcxNDg0LCJoZWlnaHQiOjEuNDcxMTAxNTIyNDQ1Njc4N319LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjkwLjAwMzExMjc5Mjk2ODc1LCJsZWZ0Ijo1MS40Nzc2NTczMTgxMTUyMzQsIndpZHRoIjozOC42NzY0OTg0MTMwODU5NCwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo5MS42MzgyNTIyNTgzMDA3OCwibGVmdCI6NTEuNDc3NjUzNTAzNDE3OTcsIndpZHRoIjozOC42NzU4MzQ2NTU3NjE3MiwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjksImJib3giOnsidG9wIjo2LjQ2ODkzNTk2NjQ5MTY5OSwibGVmdCI6OS44NDAxMzM2NjY5OTIxODgsIndpZHRoIjozOC42ODYzMjUwNzMyNDIxOSwiaGVpZ2h0IjoxNi4yMTIxMjU3NzgxOTgyNDJ9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI5In19LHsib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkpvYWNoaW0iLCJmYW1pbHkiOiJKYW5zZW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiR2F2aW4gTC4iLCJmYW1pbHkiOiJTaW1wc29uIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikdlc2EgQS4iLCJmYW1pbHkiOiJXZXloZW5tZXllciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJMYXVyYSBILiIsImZhbWlseSI6IkjDpHJrw7ZuZW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW5kcmV3IE0uIiwiZmFtaWx5IjoiUGF0ZXJzb24ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUGF1bCBBLiBkZWwiLCJmYW1pbHkiOiJHaW9yZ2lvIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ill2ZXMgVC4iLCJmYW1pbHkiOiJQcmFpcmllIn19XSwiaWRzIjp7Imlzc24iOlsiMTc1OC02NzhYIiwiMTc1OC02Nzk4Il0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDM4L3M0MTU1OC0wMjQtMDIwNTgtMyIsIm9wZW5hbGV4IjoiVzQ0MDAyMDgyMDAiLCJleGEiOiJodHRwczovL3d3dy5uYXR1cmUuY29tL2FydGljbGVzL3M0MTU1OC0wMjQtMDIwNTgtMz9lcnJvcj1jb29raWVzX25vdF9zdXBwb3J0ZWQmY29kZT1hNzRhMjVmZi1hNThkLTRkZjgtOTM2Zi0yNjJlMmUzZTkxOTgifSwidGl0bGUiOiJDbGltYXRlLWRyaXZlbiBkZW94eWdlbmF0aW9uIG9mIG5vcnRoZXJuIGxha2VzIiwiYWJzdHJhY3QiOiJBYnN0cmFjdCBPeHlnZW4gZGVwbGV0aW9uIGNvbnN0aXR1dGVzIGEgbWFqb3IgdGhyZWF0IHRvIGxha2UgZWNvc3lzdGVtcyBhbmQgdGhlIHNlcnZpY2VzIHRoZXkgcHJvdmlkZS4gTW9zdCBvZiB0aGUgd29ybGTigJlzIGxha2VzIGFyZSBsb2NhdGVkICZndDs0NcKwIE4sIHdoZXJlIGFjY2VsZXJhdGVkIGNsaW1hdGUgd2FybWluZyBhbmQgZWxldmF0ZWQgY2FyYm9uIGxvYWRzIG1pZ2h0IHNldmVyZWx5IGluY3JlYXNlIHRoZSByaXNrIG9mIGh5cG94aWEsIGJ1dCB0aGlzIGhhcyBub3QgYmVlbiBzeXN0ZW1hdGljYWxseSBleGFtaW5lZC4gSGVyZSBhbmFseXNpcyBvZiAyLjYgbWlsbGlvbiB3YXRlciBxdWFsaXR5IG9ic2VydmF0aW9ucyBmcm9tIDgsMjg4IGxha2VzIHNob3dzIHRoYXQgYmV0d2VlbiAxOTYwIGFuZCAyMDIyLCBtb3N0IG5vcnRoZXJuIGxha2VzIGV4cGVyaWVuY2VkIHJhcGlkIGRlb3h5Z2VuYXRpb24gc3Ryb25nbHkgbGlua2VkIHRvIGNsaW1hdGUtZHJpdmVuIHByb2xvbmdhdGlvbiBvZiBzdW1tZXIgc3RyYXRpZmljYXRpb24uIE94eWdlbiBsZXZlbHMgZGV0ZXJpb3JhdGVkIG1vc3QgaW4gc21hbGwgbGFrZXMgKCZsdDsxMCBoYSkgb3dpbmcgdG8gdGhlaXIgZ3JlYXRlciB2b2x1bWV0cmljIG94eWdlbiBkZW1hbmQgYW5kIHN1cmZhY2Ugd2FybWluZyByYXRlcywgd2hpbGUgdGhlIGxhcmdlc3QgbGFrZXMgZ2FpbmVkIG94eWdlbiB1bmRlciBtaW5pbWFsIHN0cmF0aWZpY2F0aW9uIGNoYW5nZXMgYW5kIGltcHJvdmVkIGFlcmF0aW9uIGF0IHNwcmluZyBvdmVydHVybnMuIFNlYXNvbmFsIG94eWdlbiBjb25zdW1wdGlvbiByYXRlcyBkZWNsaW5lZCwgZGVzcGl0ZSB3aWRlc3ByZWFkIGJyb3duaW5nLiBQcm9saWZlcmF0aW5nIGFub3hpYSBlbmhhbmNlZCBzZWFzb25hbCBpbnRlcm5hbCBsb2FkaW5nIG9mIEMsIFAgYW5kIE4gYnV0IGRlcGxldGVkIFAgbG9uZy10ZXJtLCBpbmRpY2F0aW5nIHRoYXQgZGVveHlnZW5hdGlvbiBjYW4gZXhoYXVzdCByZWRveC1zZW5zaXRpdmUgZnJhY3Rpb25zIG9mIHNlZGltZW50IG51dHJpZW50IHJlc2Vydm9pcnMuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjEwMzgvczQxNTU4LTAyNC0wMjA1OC0zIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAzOC9zNDE1NTgtMDI0LTAyMDU4LTMiLCJpc3N1ZWREYXRlIjoiMjAyNC0wNy0wMSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTEiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6Ik5hdHVyZSBDbGltYXRlIENoYW5nZSIsImlzc3VlIjo4LCJ2b2x1bWUiOjE0LCJpbXBhY3RGYWN0b3IiOiIxNS4wMiJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjgzMiwibGFzdFBhZ2UiOjgzOH19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJOYXR1cmUgUG9ydGZvbGlvIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo4MCwiY2l0ZWRCeUNvdW50IjozN319LCJxdW90ZSI6IkFlcm9iaWMgcmVzcGlyYXRpb24gY2FuIGRlcGxldGUgb3h5Z2VuIGNvbmNlbnRyYXRpb25zIGF0IGRlcHRoIHdoZW4gc2Vhc29uYWwgc3RyYXRpZmljYXRpb24gcHJldmVudHMgYWVyYXRpb24gb2YgdGhlIHdhdGVyIGNvbHVtbls1XShodHRwczovL3d3dy5uYXR1cmUuY29tL2FydGljbGVzL3M0MTU1OC0wMjQtMDIwNTgtMyNyZWYtQ1I1KS4gVGhpcyBkZXRyaW1lbnRhbCBjb25kaXRpb24sIHRlcm1lZCBoeXBveGlhIChET%2BKAiTzigIky4oCJbWfigIls4oiSMSksIGRpbWluaXNoZXMgdGhlIGNvbGQtd2F0ZXIgaGFiaXRhdCBvZiBhcXVhdGljIGxpZmUsIGluY2x1ZGluZyBjb21tZXJjaWFsbHkgaW1wb3J0YW50IGZpc2hbNl0oaHR0cHM6Ly93d3cubmF0dXJlLmNvbS9hcnRpY2xlcy9zNDE1NTgtMDI0LTAyMDU4LTMjcmVmLUNSNikgYW5kIGNhbiBjYXVzZSBtYXNzIG1vcnRhbGl0eSBpZiBwZXJzaXN0ZW50WzddKGh0dHBzOi8vd3d3Lm5hdHVyZS5jb20vYXJ0aWNsZXMvczQxNTU4LTAyNC0wMjA1OC0zI3JlZi1DUjcpLiBBbm94aWEgKERP4oCJPOKAiTAuNeKAiW1n4oCJbOKIkjEpIHByb21vdGVzIHRoZSByZWR1Y3RpdmUgZGlzc29sdXRpb24gb2YgbWluZXJhbC1ib3VuZCBjYXJib24gYW5kIHBob3NwaG9ydXNbOF0oaHR0cHM6Ly93d3cubmF0dXJlLmNvbS9hcnRpY2xlcy9zNDE1NTgtMDI0LTAyMDU4LTMjcmVmLUNSOCksIFs5XShodHRwczovL3d3dy5uYXR1cmUuY29tL2FydGljbGVzL3M0MTU1OC0wMjQtMDIwNTgtMyNyZWYtQ1I5KSB3aGljaCBmdWVscyBoYXJtZnVsIGFsZ2FsIGJsb29tc1sxMF0oaHR0cHM6Ly93d3cubmF0dXJlLmNvbS9hcnRpY2xlcy9zNDE1NTgtMDI0LTAyMDU4LTMjcmVmLUNSMTApIGFuZCBsaW1pdHMgbWljcm9iaWFsIG94aWRhdGlvbiBvZiBtZXRoYW5lLCBhIHBvdGVudCBncmVlbmhvdXNlIGdhc1sxMV0oaHR0cHM6Ly93d3cubmF0dXJlLmNvbS9hcnRpY2xlcy9zNDE1NTgtMDI0LTAyMDU4LTMjcmVmLUNSMTEpLiJ9XQ%3D%3D.

Loktak Lake in Manipur and Rewalsar Lake in Himachal Pradesh reveal inland parallels. Loktak's phumdis floating algal mats harbor Microcystis, causing anoxia and sangai deer habitat loss, with fisheries declining 50% (Nair & Nayak, 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hdnlhIFZpa3JhbWFuIiwiZmFtaWx5IjoiTmFpciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQcmF0ZWVwIEt1bWFyIiwiZmFtaWx5IjoiTmF5YWsifX1dLCJpZHMiOnsiaXNzbiI6WyIyMjk2LTc3NDUiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzODkvZm1hcnMuMjAyMy4xMDg3Mjk2Iiwib3BlbmFsZXgiOiJXNDM4MjcyMzEzMyJ9LCJ0aXRsZSI6IlVuY292ZXJpbmcgd2F0ZXIgcXVhbGl0eSBhbmQgZXZhbHVhdGluZyB2dWxuZXJhYmlsaXRpZXMgb2Ygc21hbGwtc2NhbGUgZmlzaGVyaWVzIGluIENoaWxpa2EgTGFnb29uLCBJbmRpYSIsImFic3RyYWN0IjoiU21hbGwtc2NhbGUgZmlzaGVyaWVzIChTU0ZzKSBwbGF5IGEgc3Ryb25nIHJvbGUgaW4gc3VzdGFpbmluZyBtaWxsaW9ucyBvZiBsaXZlbGlob29kcywgZm9vZCBzZWN1cml0eSwgbnV0cml0aW9uLCBhbmQgaW5jb21lIGdsb2JhbGx5IGJ1dCB0aGUgZmlzaGVycyBlbmdhZ2VkIGluIHRoaXMgc2VjdG9yIHNpbXVsdGFuZW91c2x5IGV4cGVyaWVuY2UgaGlnaCBsZXZlbHMgb2YgdnVsbmVyYWJpbGl0eSBhbmQgcHJvY2Vzc2VzIG9mIG1hcmdpbmFsaXNhdGlvbi4gU2V2ZXJhbCBmYWN0b3JzIGFyZSBhdHRyaWJ1dGVkIHRvIHRoZSBtdWx0aWRpbWVuc2lvbmFsIHZ1bG5lcmFiaWxpdGllcyB0aGUgc21hbGwtc2NhbGUgZmlzaGVycyBleHBlcmllbmNlLCBzcGFubmluZyBib3RoIG5hdHVyYWwgKGUuZy4sIG5hdHVyYWwgZGlzYXN0ZXJzLCBlY29zeXN0ZW0gY2hhbmdlKSBhbmQgYW50aHJvcG9nZW5pYyAoZS5nLiwgcG9saWN5IGNoYW5nZSwgaHlkcm9sb2dpY2FsIGludGVydmVudGlvbnMsIGFxdWFjdWx0dXJlKSBwcmVzc3VyZXMuIFdoaWxlIHRoZXJlIGlzIG11Y2ggbGl0ZXJhdHVyZSBvbiB2YXJpb3VzIG5hdHVyYWwgYW5kIGh1bWFuIGRyaXZlcnMgb2YgdnVsbmVyYWJpbGl0eSBpbiBzbWFsbC1zY2FsZSBmaXNoZXJ5IGNvbW11bml0aWVzLCBhbiBhYnNlbmNlIG9mIHJlc2VhcmNoIGNvbm5lY3RpbmcgdnVsbmVyYWJpbGl0eSB3aXRoIHdhdGVyIHF1YWxpdHkgaXMgZXZpZGVudC4gRmlzaGVyIGNvbW11bml0aWVzIG9mdGVuIHRhbGsgYWJvdXQgZmlzaCBpbiByZWxhdGlvbiB0byB0aGUgaGVhbHRoIG9mIHRoZWlyIGFxdWF0aWMgaGFiaXRhdHMgd2hlcmVpbiB3YXRlciBxdWFsaXR5IGlzIHNlZW4gYXMgYSBrZXkgcGFyYW1ldGVyLiBUaGUgbGluayBiZXR3ZWVuIGhlYWx0aHkgZmlzaCBhbmQgZ29vZCBxdWFsaXR5IHdhdGVyIGhhcyBzaWduaWZpY2FudCBpbXBsaWNhdGlvbnMgZm9yIHN0cm9uZyBhbmQgdmlhYmxlIGZpc2hpbmcgY29tbXVuaXRpZXMuIFRoaXMgcGFwZXIgZXhhbWluZXMgdGhlc2UgbGlua3MgZnVydGhlciBieSBmb2N1c2luZyBvbiB0aGUgbmF0dXJlIG9mIHZ1bG5lcmFiaWxpdGllcyBjYXVzZWQgYnkgd2F0ZXIgcXVhbGl0eSBjaGFuZ2VzIGluIHRoZSBzbWFsbC1zY2FsZSBmaXNoZXJ5IHN5c3RlbSBvZiBDaGlsaWthIExhZ29vbiBpbiBJbmRpYS4gV2UgdW5kZXJ0YWtlIGRldGFpbGVkIGFuYWx5c2lzIG9mIHRoZSBpbnZhc2l2ZSBzaHJpbXAgYXF1YWN1bHR1cmUgYWN0aXZpdGllcyBhbmQgaHlkcm9sb2dpY2FsIGludGVydmVudGlvbnMgZm9yIG9wZW5pbmcgb2YgYSBsYWdvb24gaW5sZXQgd2l0aCB0aGUgQmF5IG9mIEJlbmdhbCBhcyB0d28gZG9taW5hbnQgZHJpdmVycyBhZHZlcnNlbHkgaW1wYWN0aW5nIHdhdGVyIHF1YWxpdHkgYW5kIGluY3JlYXNpbmcgdnVsbmVyYWJpbGl0aWVzIG9mIHRoZSBlbnRpcmUgc21hbGwtc2NhbGUgZmlzaGVyaWVzIHNvY2lhbC1lY29sb2dpY2FsIHN5c3RlbS4gT3VyIGFuYWx5c2lzIHN1Z2dlc3RzIHRoYXQgdGhlcmUgYXJlIHN0cm9uZyBpbnRlcmNvbm5lY3Rpb25zIGJldHdlZW4gY2hhbmdlcyBpbiB3YXRlciBxdWFsaXR5IGFuZCB0aGUgbGV2ZWxzIG9mIHZ1bG5lcmFiaWxpdGllcyBpbiB0aGUgU1NGcyBvZiBDaGlsaWthIExhZ29vbi4gUG9sbHV0YW50cyBzdWNoIGFzIHBlc3RpY2lkZXMsIGFuZCBvcmdhbmljIGNvbXBvdW5kcyBhY2N1bXVsYXRlIGluIGZpc2ggdGlzc3VlcyBhbmQgYWZmZWN0IHRoZWlyIGdyb3d0aCwgcmVwcm9kdWN0aW9uLCBhbmQgb3ZlcmFsbCBoZWFsdGguIFRoaXMgbGVkIHRvIGRlY2xpbmVzIGluIGZpc2ggcG9wdWxhdGlvbnMsIG1ha2luZyBpdCBtb3JlIGRpZmZpY3VsdCBmb3IgZmlzaGVycyB0byBtYWtlIGEgbGl2aW5nLiBJbiBhZGRpdGlvbiB0byBkaXJlY3QgaW1wYWN0cyBvbiBmaXNoIHBvcHVsYXRpb25zLCBwb29yIHdhdGVyIHF1YWxpdHkgYWxzbyBoYXMgaW5kaXJlY3QgZWZmZWN0cyBvbiB0aGUgc29jaWFsIGFuZCBlY29ub21pYyB2dWxuZXJhYmlsaXR5IG9mIFNTRnMuIEZvciBleGFtcGxlLCBjb250YW1pbmF0aW9uIG9mIHdhdGVyIHNvdXJjZXMgbGVkIHRvIHRoZSByZWR1Y2VkIG51bWJlciBvZiBmaXNoIHNwZWNpZXMgcmVkdWNpbmcgdGhlIGFtb3VudCBvZiB0aW1lIGZpc2hlcnMgY2FuIHNwZW5kIG9uIGZpc2hpbmcgYWN0aXZpdGllcy4gVGhpcyBhbHNvIGFmZmVjdGVkIHRoZSBtYXJrZXRhYmlsaXR5IG9mIGZpc2ggcHJvZHVjdHMsIHJlZHVjZWQgaW5jb21lIGFuZCBpbmNyZWFzZWQgcG92ZXJ0eS4gVG8gZnVsbHkgdW5kZXJzdGFuZCB0aGUgaW50ZXJjb25uZWN0aW9ucyBiZXR3ZWVuIHdhdGVyIHF1YWxpdHkgYW5kIHZ1bG5lcmFiaWxpdHkgaW4gU1NGcyBpbiBDaGlsaWthIExhZ29vbiwgaXQgaXMgaW1wb3J0YW50IHRvIGNvbnNpZGVyIGJvdGggZW52aXJvbm1lbnRhbCBhbmQgc29jaWFsIGZhY3RvcnMsIGFzIHdlbGwgYXMgdGhlIGNvbXBsZXggZmVlZGJhY2sgbG9vcHMgYmV0d2VlbiB0aGVzZSBmYWN0b3JzLiBUaGUgc3R1ZHkgaGVscHMgaW4gYnJpZGdpbmcgYSBjcnVjaWFsIGdhcCBpbiBvdXIgdW5kZXJzdGFuZGluZyBvZiB0aGUgcm9sZSBvZiB3YXRlciBxdWFsaXR5IGluIHZ1bG5lcmFiaWxpdHkgYW5hbHlzaXMgd2l0aGluIHJlc291cmNlIGRlcGVuZGVudCBjb21tdW5pdGllcy4gV2UgY29uY2x1ZGUgd2l0aCBrZXkgaW5zaWdodHMgb24gcG9zc2libGUgY29waW5nIHJlc3BvbnNlcyBhbmQgYWRhcHRpdmUgY2FwYWNpdHkgbmVjZXNzYXJ5IGZvciB0aGUgc21hbGwtc2NhbGUgZmlzaGVyaWVzIGNvbW11bml0aWVzIHRvIHRyYW5zaXRpb24gdG93YXJkIHZpYWJpbGl0eS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMzM4OS9mbWFycy4yMDIzLjEwODcyOTYiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vd3d3LmZyb250aWVyc2luLm9yZy9hcnRpY2xlcy8xMC4zMzg5L2ZtYXJzLjIwMjMuMTA4NzI5Ni9wZGYiLCJpc3N1ZWREYXRlIjoiMjAyMy0wNi0yOSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZyb250aWVycyBpbiBNYXJpbmUgU2NpZW5jZSIsInZvbHVtZSI6MTAsImltcGFjdEZhY3RvciI6IjQuMjgifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRnJvbnRpZXJzIE1lZGlhIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo1MSwiY2l0ZWRCeUNvdW50Ijo1fX0sInF1b3RlIjoiV2F0ZXIgcXVhbGl0eSBvZiBDaGlsaWthIExhZ29vbiBoYXMgZGVncmFkZWQgZHVlIHRvIGFudGhyb3BvZ2VuaWMgaW50ZXJ2ZW50aW9ucyBhbmQgbmF0dXJhbCBmb3JjZXMgb2YgY2hhbmdlLiBWYXJpb3VzIGFjdGl2aXRpZXMgaW4gdGhlIENoaWxpa2EgTGFnb29uLCBzdWNoIGFzIGluZHVzdHJpYWwgd2FzdGV3YXRlciBkaXNwb3NhbCwgc2V3YWdlIGR1bXBpbmcsIGFxdWFjdWx0dXJlLCBoeWRyb2xvZ2ljYWwgaW50ZXJ2ZW50aW9ucywgYW5kIGN5Y2xvbmVzLCBhcmUgcHJvZHVjaW5nIHNhbGluaXR5IGNoYW5nZXMsIHNlZGltZW50IGRlcG9zaXRpb24sIG51dHJpZW50IGVucmljaG1lbnQsIGV1dHJvcGhpY2F0aW9uLCBhbmQgZGVhZCB6b25lcywgYXMgc3RhdGVkLiBGaXNoIHN1cnZpdmFsIGlzIGF0IHJpc2sgYXMgYSByZXN1bHQgb2YgdGhlIHNpZ25pZmljYW50IHJlZHVjdGlvbiBpbiBlbnZpcm9ubWVudGFsIGNpcmN1bXN0YW5jZXMsIHdoaWNoIGRpbWluaXNoZXMgZmlzaGVybWVu4oCZcyBpbmNvbWUgYW5kIHB1c2hlcyB0aGVtIG91dCBvZiBzb2NpZXR5LiBUaGVzZSBjYXVzZXMgYWZmZWN0aW5nIHRoZSB3YXRlciBxdWFsaXR5IG9mIHRoZSBMYWdvb24gZGVwaWN0IG1hbnkgZmFjZXMgb2YgdnVsbmVyYWJpbGl0eSBpbiBTU0YgY29tbXVuaXRpZXMuIEFzIHRoZSBDaGlsaWthIExhZ29vbiBlbnZpcm9ubWVudCBpcyBleHBlY3RlZCB0byB1bmRlcmdvIGluY3JlYXNpbmdseSBkcmFzdGljIGNoYW5nZXMgKHN1Y2ggYXMgY2F0Y2ggY29tcG9zaXRpb24sIGNhdGNoIGNhcGFjaXR5LCBiaW9kaXZlcnNpdHkgdmFyaWF0aW9ucywgd2F0ZXIgcXVhbGl0eSwgYW5kIGZpc2hlcnkgcmV2ZW51ZXMpLCBpbnRlcm5hdGlvbmFsIHBvbGljeSBoYXMgYmVjb21lIG1vcmUgaW50ZXJlc3RlZCBpbiBhZGFwdGF0aW9uIG1lYXN1cmVzLiBHaXZlbiBob3cgZGl2ZXJzZSBTU0YgZWNvc3lzdGVtcyBhcmUgZXhwZWN0ZWQgdG8gYmUgYmV0d2VlbiBjb250aW5lbnRzLCB0aGVyZSBpcyBubyDigJxvbmUgc2l6ZSBmaXRzIGFsbOKAnSBhcHByb2FjaC4gVGhlIHBhcGVyIG91dGxpbmVkIHNldmVyYWwgY29waW5nIGFuZCBhZGFwdGF0aW9uIG1lY2hhbmlzbXMgdG8gaW1wcm92ZSB3YXRlciBxdWFsaXR5IGFuZCBjb250aW51ZSBmaXNoaW5nIGluIGEgc3VzdGFpbmFibGUgd2F5LiBBZGFwdGF0aW9uIHN0cmF0ZWdpZXMgY2FuIG9jY2FzaW9uYWxseSBoYXZlIHVuZm9yZXNlZW4gY29uc2VxdWVuY2VzIGZvciBmaXNoZXJpZXMgYW5kIGZpc2hpbmcgY29tbXVuaXRpZXMuIEFueSB1bmV4cGVjdGVkIGNvbnNlcXVlbmNlcyBvZiBhZGFwdGF0aW9uIGNhbiBiZSBtaXRpZ2F0ZWQgdGhyb3VnaCBsb25nLXRlcm0gcGxhbm5pbmcgYW5kIGlkZW50aWZpY2F0aW9uIG9mIGZ1dHVyZSBvcHRpb25zIHRocm91Z2ggc2NlbmFyaW8gYW5hbHlzaXMuIFRoZSBmaW5kaW5ncyBvZiB0aGlzIHBhcGVyIGNhbiBiZSB1dGlsaXplZCB0byBkZXZlbG9wIHBvbGljaWVzIHRvIHByb3RlY3Qgd2F0ZXIgcXVhbGl0eSBhbmQgU1NGcyBpbiBhIHZhcmlldHkgb2YgY29hc3RhbCBsYWdvb24gc2V0dGluZ3MuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjIyLjI4NTAzNjA4NzAzNjEzMywibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ5MzA1MzQzNjI3OTMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjIzLjc3MzUxOTUxNTk5MTIxLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDgzMzY0MTA1MjI0NjEsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjI1LjI1NTU4NDcxNjc5Njg3NSwibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ4OTU1OTE3MzU4Mzk4NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjYuNzQ0MDY4MTQ1NzUxOTUzLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNTA2ODA1NDE5OTIxODc1LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyOC4yMjYxMzMzNDY1NTc2MTcsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40OTA0NjMyNTY4MzU5NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjkuNzE0NjE2Nzc1NTEyNjk1LCJsZWZ0Ijo4LjMzMTU5MjU1OTgxNDQ1Mywid2lkdGgiOjQwLjQ5NDA5ODY2MzMzMDA4LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozMS4xOTY2ODM4ODM2NjY5OTIsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40OTQzOTIzOTUwMTk1MywiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MzIuNjg1MTY1NDA1MjczNDQsImxlZnQiOjguMzMxNTk2Mzc0NTExNzE5LCJ3aWR0aCI6NDAuNDg3NDQ1ODMxMjk4ODMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM0LjE2NzIzMjUxMzQyNzczNCwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40OTc3NDU1MTM5MTYwMTYsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM1LjY1NTcxNTk0MjM4MjgxLCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ5ODYzODE1MzA3NjE3LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozNy4xMzc3ODMwNTA1MzcxMSwibGVmdCI6MTAuNjg1MTg2Mzg2MTA4Mzk4LCJ3aWR0aCI6MzguMTQxMTA1NjUxODU1NDcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM4LjYyNjI2NjQ3OTQ5MjE5LCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4NzE1MjA5OTYwOTM3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDAuMTA4MzMzNTg3NjQ2NDg0LCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4OTg1NjcxOTk3MDcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQxLjU5NjgxMzIwMTkwNDMsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDg4NjY2NTM0NDIzODMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQzLjA3ODg4MDMxMDA1ODU5NCwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40ODE4NjExMTQ1MDE5NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDQuNTY3MzYzNzM5MDEzNjcsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDgzMDU4OTI5NDQzMzYsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQ2LjA0OTQzMDg0NzE2Nzk3LCJsZWZ0Ijo4LjMzMzEwODkwMTk3NzUzOSwid2lkdGgiOjQwLjUwMDc0NzY4MDY2NDA2LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo0Ny41Mzc5MTQyNzYxMjMwNSwibGVmdCI6OC4zMzMxMDg5MDE5Nzc1MzksIndpZHRoIjo0MC40OTU2MDU0Njg3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDkuMDE5OTgxMzg0Mjc3MzQ0LCJsZWZ0Ijo4LjMzMzEwNjA0MDk1NDU5LCJ3aWR0aCI6NDAuNTA2ODE2ODY0MDEzNjcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUwLjUwODQ2NDgxMzIzMjQyLCJsZWZ0Ijo4LjMzMzEwNjA0MDk1NDU5LCJ3aWR0aCI6NDAuNDg4NTA2MzE3MTM4NjcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUxLjk5MDUyODEwNjY4OTQ1LCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDk3NzI2NDQwNDI5NjksImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUzLjQ3OTAxMTUzNTY0NDUzLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6MzMuMTU5NDQ2NzE2MzA4NTk0LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo1NC45NjEwNzg2NDM3OTg4MywibGVmdCI6MTAuNjgzNjcwMDQzOTQ1MzEyLCJ3aWR0aCI6MzguMTM2NTk2Njc5Njg3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NTYuNDQ5NTYyMDcyNzUzOTA2LCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDg3MTQ0NDcwMjE0ODQ0LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEzIn19XQ%3D%3D. Rewalsar experienced Microcystis blooms tied to cattle deaths and fishery bans, amplifying poverty (Kumar & Mahajan, 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlBhd2FuIiwiZmFtaWx5IjoiS3VtYXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW1icmlzaCBLdW1hciIsImZhbWlseSI6Ik1haGFqYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyNTIzLTM5NjMiLCIyNTIzLTM5NzEiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMDcvczQyNDUyLTAyMC0zMDgyLTgiLCJtYWciOiIzMDM1OTYwMjI4Iiwib3BlbmFsZXgiOiJXMzAzNTk2MDIyOCJ9LCJ0aXRsZSI6IlRyb3BoaWMgc3RhdHVzIGFuZCBpdHMgcmVndWxhdGluZyBmYWN0b3IgZGV0ZXJtaW5hdGlvbiBhdCB0aGUgUmV3YWxzYXIgTGFrZSwgbm9ydGh3ZXN0IEhpbWFsYXlhIChIUCksIEluZGlhLCBiYXNlZCBvbiBzZWxlY3RlZCBwYXJhbWV0ZXJzIGFuZCBtdWx0aXZhcmlhdGUgc3RhdGlzdGljYWwgYW5hbHlzaXMiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAwNy9zNDI0NTItMDIwLTMwODItOCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9saW5rLnNwcmluZ2VyLmNvbS9jb250ZW50L3BkZi8xMC4xMDA3L3M0MjQ1Mi0wMjAtMzA4Mi04LnBkZiIsImlzc3VlZERhdGUiOiIyMDIwLTA2LTIyIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiU04gQXBwbGllZCBTY2llbmNlcyIsImlzc3VlIjo3LCJ2b2x1bWUiOjIsImltcGFjdEZhY3RvciI6IjcuMzUifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiU3ByaW5nZXIgTmF0dXJlIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo0MCwiY2l0ZWRCeUNvdW50IjoxOX19LCJxdW90ZSI6IkNoYW5naW5nIHF1YWxpdHkgaGFzIGNyZWF0ZWQgdGhlIHdhdGVyIHNjYXJjaXR5IGFuZCAgYSBodWdlIG1hc3Mgb2YgcG9wdWxhdGlvbiB0aHVzIGJlZW4gZGVwcml2ZWQgZnJvbSB1c2luZyAgdGhlIGZyZXNod2F0ZXIgZm9yIHZhcmlvdXMgcHVycG9zZXMgWzMsIDRdLiBIb3dldmVyLCB0aGUgIG9ibm94aW91cyBtYXRlcmlhbHMgdGhhdCBzdGF5IGluIHdhdGVyIGJvZGllcyBmb3IgYSBsb25nLXRpbWUgcGVyaW9kIGJyaW5nIGNoYW5nZXMgaW4gdGhlIHBoeXNpb2NoZW1pY2FsIGNoYXItYWN0ZXJpc3RpY3MgYXMgd2VsbCBhcyB0aGUgbmF0dXJhbCBzY2VuYXJpbyBvZiB0aGUgbGFrZXMgWzVdLiAgVGhlIEhpbWFsYXlhbiBsYWtlcyBhcmUgdW5kZXIgc3RyZXNzIGR1ZSB0byBtaXNtYW5hZ2VkIGh1bWFuIGFjdGl2aXRpZXMsIHdoaWNoIGluY3JlYXNlIG51dHJpZW50cyBhbmQgcG9sbHV0aW9uIGluIHRlcm1zIG9mIGhlYXZ5IG1ldGFscyB0b3hpY2l0eSBhbmQgYmFjdGVyaWFsIGNvbnRhbWluYXRpb24gIFs24oCTOF0uIFRoZSBtYW5hZ2VtZW50IGFuZCBwcmVzZXJ2YXRpb24gb2YgdGhlIGxha2VzIGF0ICBwcmVzZW50IGhhdmUgYmVjb21lIGEgZ3JlYXQgY2hhbGxlbmdlIGZvciBlbnZpcm9ubWVudGFsLWlzdHMgYW5kIG1hbmFnZXJzIFs1LCA5XS4gVGhlIGV1dHJvcGhpY2F0aW9uIGlzIGFsc28gYSBtYWluIHByb2JsZW0gb2YgdGhlICBmcmVzaHdhdGVyIGxha2VzLCBhbmQgaXQgaXMgc3BlY2lmaWVkIGZvciB0aGUgbGFrZXMgaGF2aW5nICBudXRyaWVudCBlbnJpY2htZW50LiBJdCBjYXVzZXMgZXhjZXNzaXZlIGdyb3d0aCBvZiBhbGdhZSAgYW5kIGlzIGNvbnNpZGVyZWQgdG8gYmUgcmVzcG9uc2libGUgZm9yIGRlZ3JhZGluZyB0aGUgIHF1YWxpdGF0aXZlIG5hdHVyZSBvZiB0aGUgbGFrZXMgWzUsIDEwXS4gVGhlIGV1dHJvcGhpY2F0aW9uICBpcyBmb2xsb3dlZCBieSB0aGUgYWxnYWwgYmxvb20sIHdoaWNoIGdlbmVyYXRlcyB0b3hpbiwgIHdvcnNlbnMgd2F0ZXIgcXVhbGl0eSwgZGVwbGV0ZXMgZGlzc29sdmVkIG94eWdlbiAoRE8pICogUGF3YW4gS3VtYXIsIHBhd2FuLmV2c0BnbWFpbC5jb20gfCAxRGVwYXJ0bWVudCBvZiBFbnZpcm9ubWVudGFsIFNjaWVuY2VzLCBDZW50cmFsIFVuaXZlcnNpdHkgb2YgSGltYWNoYWwgUHJhZGVzaCwgRGhhcmFtc2hhbGEgMTc2MjE1LCBJbmRpYS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2OC41Nzg0NjA2OTMzNTkzOCwibGVmdCI6OC41NzE0MTg3NjIyMDcwMzEsIndpZHRoIjo0MC4zMjc1MTQ2NDg0Mzc1LCJoZWlnaHQiOjEuNTE5ODU1MjYwODQ4OTk5fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NzAuMTU4OTk2NTgyMDMxMjUsImxlZnQiOjguNTY5NzM4Mzg4MDYxNTIzLCJ3aWR0aCI6NDAuMzU0NzI0ODg0MDMzMiwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjcxLjczOTU0NzcyOTQ5MjE5LCJsZWZ0Ijo4LjU2OTczODM4ODA2MTUyMywid2lkdGgiOjQwLjM0OTY5MzI5ODMzOTg0NCwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjczLjMyMDA5MTI0NzU1ODYsImxlZnQiOjguNTY4MDU5OTIxMjY0NjQ4LCJ3aWR0aCI6NDAuMDIwMDg4MTk1ODAwNzgsImhlaWdodCI6MS41MTk4NTUyNjA4NDg5OTl9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo3NC45MDA2NDIzOTUwMTk1MywibGVmdCI6OC41NjgwNTk5MjEyNjQ2NDgsIndpZHRoIjozOS45OTk5Mjc1MjA3NTE5NSwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjc2LjQ4MTE4NTkxMzA4NTk0LCJsZWZ0Ijo4LjU2ODA1NzA2MDI0MTcsIndpZHRoIjo0MC4zNTcwNzQ3Mzc1NDg4MywiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjc4LjA2MTcyOTQzMTE1MjM0LCJsZWZ0Ijo4LjU2OTczNjQ4MDcxMjg5LCJ3aWR0aCI6NDAuMzUzNzEzOTg5MjU3ODEsImhlaWdodCI6MS41MTk4NTUyNjA4NDg5OTl9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2MC42NzU3Mjc4NDQyMzgyOCwibGVmdCI6NTEuNDI3MTY1OTg1MTA3NDIsIndpZHRoIjo0MC4zNzg1NjY3NDE5NDMzNiwiaGVpZ2h0IjoxOC45MDU4NTUxNzg4MzMwMDh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo4MS45MjczMzc2NDY0ODQzOCwibGVmdCI6OC40MDE0OTc4NDA4ODEzNDgsIndpZHRoIjo3My45NTQ2OTY2NTUyNzM0NCwiaGVpZ2h0IjoyLjY3ODc2MjY3NDMzMTY2NX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEifX1d. Common threads include sewage phosphorus, warming stratification, and toxin feedbacks eroding economies valued at billions (Paerl et al., 2019)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhhbnMgVy4iLCJmYW1pbHkiOiJQYWVybCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLYXJsIEUuIiwiZmFtaWx5IjoiSGF2ZW5zIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hdGhhbiBTLiIsImZhbWlseSI6IkhhbGwifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiVGltb3RoeSBHLiIsImZhbWlseSI6Ik90dGVuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1lbmd5dWFuIiwiZmFtaWx5IjoiWmh1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhhaSIsImZhbWlseSI6Ilh1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikd1YW5nd2VpIiwiZmFtaWx5IjoiWmh1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkJvcWlhbmciLCJmYW1pbHkiOiJRaW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIxMzIzLTE2NTAiLCIxNDQ4LTYwNTkiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwNzEvbWYxODM5MiIsIm1hZyI6IjI5NDIzMjE1NDIiLCJvcGVuYWxleCI6IlcyOTQyMzIxNTQyIn0sInRpdGxlIjoiTWl0aWdhdGluZyBhIGdsb2JhbCBleHBhbnNpb24gb2YgdG94aWMgY3lhbm9iYWN0ZXJpYWwgYmxvb21zOiBjb25mb3VuZGluZyBlZmZlY3RzIGFuZCBjaGFsbGVuZ2VzIHBvc2VkIGJ5IGNsaW1hdGUgY2hhbmdlIiwiYWJzdHJhY3QiOiJNYW5hZ2luZyBhbmQgbWl0aWdhdGluZyB0aGUgZ2xvYmFsIGV4cGFuc2lvbiBvZiB0b3hpYyBjeWFub2JhY3RlcmlhbCBoYXJtZnVsIGFsZ2FsIGJsb29tcyAoQ3lhbm9IQUJzKSBpcyBhIG1ham9yIGNoYWxsZW5nZSBmYWNpbmcgcmVzZWFyY2hlcnMgYW5kIHdhdGVyIHJlc291cmNlIG1hbmFnZXJzLiBWYXJpb3VzIGFwcHJvYWNoZXMsIGluY2x1ZGluZyBudXRyaWVudCBsb2FkIHJlZHVjdGlvbiwgYXJ0aWZpY2lhbCBtaXhpbmcgYW5kIGZsdXNoaW5nLCBvbW5pdm9yb3VzIGZpc2ggcmVtb3ZhbCwgYWxnYWVjaWRlIGFwcGxpY2F0aW9ucyBhbmQgc2VkaW1lbnQgZHJlZGdpbmcsIGhhdmUgYmVlbiB1c2VkIHRvIHJlZHVjZSBibG9vbSBvY2N1cnJlbmNlcy4gSG93ZXZlciwgbWFuYWdlcnMgbm93IGZhY2UgdGhlIGFkZGl0aW9uYWwgY2hhbGxlbmdlIG9mIGhhdmluZyB0byBhZGRyZXNzIHRoZSBlZmZlY3RzIG9mIGNsaW1hdGUgY2hhbmdlIG9uIHdhdGVyc2hlZCBoeWRyb2xvZ2ljYWwgYW5kIG51dHJpZW50IGxvYWQgZHluYW1pY3MsIHdhdGVyIHRlbXBlcmF0dXJlLCBtaXhpbmcgcmVnaW1lIGFuZCBpbnRlcm5hbCBudXRyaWVudCBjeWNsaW5nLiBSaXNpbmcgdGVtcGVyYXR1cmVzIGFuZCBpbmNyZWFzaW5nIGZyZXF1ZW5jaWVzIGFuZCBtYWduaXR1ZGVzIG9mIGV4dHJlbWUgd2VhdGhlciBldmVudHMsIGluY2x1ZGluZyB0cm9waWNhbCBjeWNsb25lcywgZXh0cmF0cm9waWNhbCBzdG9ybXMsIGZsb29kcyBhbmQgZHJvdWdodHMsIGFsbCBwcm9tb3RlIEN5YW5vSEFCcyBhbmQgYWZmZWN0IHRoZSBlZmZpY2FjeSBvZiBlY29zeXN0ZW0gcmVtZWRpYXRpb24gbWVhc3VyZXMuIFRoZXNlIGNsaW1hdGljIGNoYW5nZXMgd2lsbCBsaWtlbHkgcmVxdWlyZSBzZXR0aW5nIHN0cmljdGVyIG51dHJpZW50IChpbmNsdWRpbmcgYm90aCBuaXRyb2dlbiBhbmQgcGhvc3Bob3J1cykgcmVkdWN0aW9uIHRhcmdldHMgZm9yIGJsb29tIGNvbnRyb2wgaW4gYWZmZWN0ZWQgd2F0ZXJzLiBJbiBhZGRpdGlvbiwgdGhlIGVmZmljYWN5IG9mIGN1cnJlbnRseSB1c2VkIG1ldGhvZHMgdG8gcmVkdWNlIEN5YW5vSEFCcyB3aWxsIG5lZWQgdG8gYmUgcmUtZXZhbHVhdGVkIGluIGxpZ2h0IG9mIHRoZSBzeW5lcmdpc3RpYyBlZmZlY3RzIG9mIGNsaW1hdGUgY2hhbmdlIHdpdGggbnV0cmllbnQgZW5yaWNobWVudC4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTA3MS9tZjE4MzkyIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2Nkci5saWIudW5jLmVkdS9kb3dubG9hZHMvMDU3NDIyODM0IiwiaXNzdWVkRGF0ZSI6IjIwMTktMDMtMjUiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTEwIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJNYXJpbmUgYW5kIEZyZXNod2F0ZXIgUmVzZWFyY2giLCJpc3N1ZSI6NSwidm9sdW1lIjo3MSwiaW1wYWN0RmFjdG9yIjoiMi43MCJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjU3OSwibGFzdFBhZ2UiOjU3OX19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJDU0lSTyBQdWJsaXNoaW5nIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjoxMDQsImNpdGVkQnlDb3VudCI6OTF9fSwicXVvdGUiOiJUaGF0IHJlbGF0aW9uc2hpcCBoYXMgc2VydmVkIGFzIHRoZSBmb3VuZGF0aW9uIGZvciBzZXR0aW5nICBudXRyaWVudCBsb2FkaW5nIHRhcmdldHMgaW4gbGFrZXMgYXJvdW5kIHRoZSB3b3JsZCwgYW5kIGNhcmVmdWxseSAgZGVzaWduZWQgZXhwZXJpbWVudHMgbWF5IGVsdWNpZGF0ZSB0byB3aGF0IGV4dGVudCB0ZW1wZXJhdHVyZSAgaW5jcmVhc2UgYWZmZWN0cyB0aGUgdGhyZXNob2xkIGF0IHdoaWNoIHRoZSBmcmVxdWVuY3kgb2Ygb2NjdXIgIHJlbmNlIG9mIEN5YW5vSEFCcyBxdWlja2x5IHJpc2VzIHdpdGggaW5jcmVhc2luZyBOIGFuZCBQLiBDb25jbHVzaW9ucyBSZWNlbnQgc3R1ZGllcyBpbmRpY2F0ZSB0aGF0IHRoZSBzeW5lcmdpc3RpYyBjb21iaW5hdGlvbiBvZiBhbnRocm9wb2dlbmljIG51dHJpZW50IGxvYWRpbmcsIHJpc2luZyB0ZW1wZXJhdHVyZXMsIGVuaGFuY2VkICB2ZXJ0aWNhbCBzdHJhdGlmaWNhdGlvbiwgaW5jcmVhc2VkIHJlc2lkZW5jZSB0aW1lIGFuZCBtb3JlICBleHRyZW1lIGNsaW1hdGljIGNvbmRpdGlvbnMgb3ZlcmFsbCB3aWxsIGZhdm91ciBjeWFub2JhY3RlcmlhbCAgZG9taW5hbmNlIGluIGEgd2lkZSByYW5nZSBvZiBhcXVhdGljIGVjb3N5c3RlbXMuIFRoZSBleHBhbiAgc2lvbiBvZiBDeWFub0hBQnMgaGFzIHNlcmlvdXMgY29uc2VxdWVuY2VzIGZvciBodW1hbiB3YXRlciAgc3VwcGxpZXMsIGZpc2hlcmllcyBhbmQgcmVjcmVhdGlvbmFsIHJlc291cmNlcy4gRWNvc3lzdGVtIHNjYWxlICBwaHlzaWNhbCBjaGVtaWNhbCBiaW90aWMgbWl0aWdhdGlvbiBzdHJhdGVnaWVzIHdpbGwgaGF2ZSB0byAgaW5jb3Jwb3JhdGUgbnV0cmllbnQgKG1vc3Qgb2Z0ZW4gYm90aCBOIGFuZCBQKSBpbnB1dCByZWR1Y3Rpb25zLiAgVGhlIG1hZ25pdHVkZXMgb2YgdGhlc2UgcmVkdWN0aW9ucyB3aWxsIG5lZWQgdG8gYmUgc3lzdGVtICBzcGVjaWZpYyBhbmQgd2lsbCBsaWtlbHkgbmVlZCB0byBiZSBhZGp1c3RlZCB3aXRoIGNoYW5naW5nIGNsaSAgbWF0aWMgY29uZGl0aW9ucy4gU3BlY2lmaWNhbGx5LCB3YXRlciBxdWFsaXR5IG1hbmFnZXJzIHdpbGwgaGF2ZSAgdG8gYWNjb21tb2RhdGUgd2FybWluZyBhbmQgZ3JlYXRlciBoeWRyb2xvZ2ljYWwgdmFyaWFiaWxpdHkgIGFuZCBleHRyZW1lbmVzcyByZXN1bHRpbmcgZnJvbSBjbGltYXRlIGNoYW5nZSBpbiBmb3JtdWxhdGluZyAgdGhlaXIgc3RyYXRlZ2llcy4gQSBrZXkgbG9uZyB0ZXJtIGNvbnRyb2wgd2UgY2FuIGV4ZXJ0IHRvIHJlZHVjZSAgdGhlIHJhdGUgYW5kIGV4dGVudCBvZiBnbG9iYWwgd2FybWluZyBpcyBjdXJiaW5nIGdyZWVuaG91c2UgZ2FzICBlbWlzc2lvbnMuIFdpdGhvdXQgdGhpcyBlc3NlbnRpYWwgc3RlcCwgZnV0dXJlIHdhcm1pbmcgdHJlbmRzLCAgaHlkcm9sb2dpY2FsIGV4dHJlbWVzIChyZWNvcmQgcmFpbmZhbGxzIGFuZCBkcm91Z2h0cykgYW5kIHRoZWlyICBlZmZlY3RzIG9uIGFxdWF0aWMgZWNvc3lzdGVtcyB3aWxsIGNvbnRpbnVlIHRvIHByb21vdGUgb3Bwb3IgIHR1bmlzdGljIEN5YW5vSEFCcy4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6MTQuMTQ5MjMzODE4MDU0MiwibGVmdCI6MTMuMTI4NTk1MzUyMTcyODUyLCJ3aWR0aCI6MzcuMDgyMTcyMzkzNzk4ODMsImhlaWdodCI6MS41MTY2MDIyNzc3NTU3MzczfX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjE1LjQ1MTUwODUyMjAzMzY5MSwibGVmdCI6MTMuMTI4NTQ0ODA3NDM0MDgyLCJ3aWR0aCI6MzcuMDg0ODMxMjM3NzkyOTcsImhlaWdodCI6MS41MTY2MDMyMzE0MzAwNTM3fX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjE2Ljc1Mzc4NDE3OTY4NzUsImxlZnQiOjEzLjEyODU0MjkwMDA4NTQ1LCJ3aWR0aCI6MzcuMDgzMjA5OTkxNDU1MDgsImhlaWdodCI6MS41MTY2MDQxODUxMDQzNzAxfX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjE4LjA1NjA1MzE2MTYyMTA5NCwibGVmdCI6MTMuMTI4NTQ0ODA3NDM0MDgyLCJ3aWR0aCI6MzYuNjE1MDc0MTU3NzE0ODQ0LCJoZWlnaHQiOjEuNTE2NjA0MTg1MTA0MzcwMX19LHsicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjoxOS40NjgzNzgwNjcwMTY2LCJsZWZ0IjoxMy4xMjg1NDQ4MDc0MzQwODIsIndpZHRoIjozNC4yOTA1MzExNTg0NDcyNjYsImhlaWdodCI6MS4yNDUwMTEyMTA0NDE1ODk0fX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjIyLjIyNjE0Mjg4MzMwMDc4LCJsZWZ0IjoxMy4xMjg1NTA1Mjk0Nzk5OCwid2lkdGgiOjcuNjQ5ODczNzMzNTIwNTA4LCJoZWlnaHQiOjEuMTM1NDgzMTQ1NzEzODA2Mn19LHsicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjoyMy43MzQ4NDk5Mjk4MDk1NywibGVmdCI6MTMuMTI4NTM1MjcwNjkwOTE4LCJ3aWR0aCI6MzcuMDg5Mjg2ODA0MTk5MjIsImhlaWdodCI6MjYuMDg4NjQwMjEzMDEyNjk1fX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNTg5In19XQ%3D%3D.

These cases underscore HABs' nexus of eutrophication, climate stressors, and socio-economic tolls, mirroring global patterns like U.S. losses ($100 million/year) from fisheries and tourism (Berdalet et al., 2017)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkVsaXNhIiwiZmFtaWx5IjoiQmVyZGFsZXQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUmFwaGFlbCBNLiIsImZhbWlseSI6Ikt1ZGVsYSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJFZCIsImZhbWlseSI6IlVyYmFuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhlbnJpayIsImZhbWlseSI6IkVuZXZvbGRzZW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTmVpbCBTLiIsImZhbWlseSI6IkJhbmFzIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkVpbGVlbiIsImZhbWlseSI6IkJyZXNuYW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTWljaGVsZSBBLiIsImZhbWlseSI6IkJ1cmZvcmQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiS2VpdGgiLCJmYW1pbHkiOiJEYXZpZHNvbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJDaHJpc3RvcGhlciBKLiIsImZhbWlseSI6IkdvYmxlciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJCZW5ndCIsImZhbWlseSI6Ikthcmxzb24ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUG8gVGVlbiIsImZhbWlseSI6IkxpbSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJMaW5jb2xuIiwiZmFtaWx5IjoiTWFjS2VuemllIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1hcmluYSIsImZhbWlseSI6Ik1vbnRyZXNvciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJWZXJhIEwuIiwiZmFtaWx5IjoiVHJhaW5lciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJHaXJlcyIsImZhbWlseSI6IlVzdXAifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiS2Vkb25nIiwiZmFtaWx5IjoiWWluIn19XSwiaWRzIjp7Imlzc24iOlsiMTA0Mi04Mjc1IiwiMjM3Ny02MTdYIl0sImlzYm4iOltdLCJkb2kiOiIxMC41NjcwL29jZWFub2cuMjAxNy4xMTEiLCJtYWciOiIyNTk3NjY0NzI3Iiwib3BlbmFsZXgiOiJXMjU5NzY2NDcyNyJ9LCJ0aXRsZSI6Ikdsb2JhbEhBQjogQSBOZXcgUHJvZ3JhbSB0byBQcm9tb3RlIEludGVybmF0aW9uYWwgUmVzZWFyY2gsIE9ic2VydmF0aW9ucywgYW5kIE1vZGVsaW5nIG9mIEhhcm1mdWwgQWxnYWwgQmxvb21zIGluIEFxdWF0aWMgU3lzdGVtcyIsImFic3RyYWN0IjoiQmVyZGFsZXQsIEVsaXNhIC4uLiBldCBhbC4tLSBTcGVjaWFsIGlzc3VlIG9uIEludGVybmF0aW9uYWwgQ29vcGVyYXRpb24gaW4gSGFybWZ1bCBBbGdhbCBCbG9vbSBTY2llbmNlLi0tIDEyIHBhZ2VzLCA2IGZpZ3VyZXMiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuNTY3MC9vY2Vhbm9nLjIwMTcuMTExIiwicmVzb3VyY2VVcmwiOiJodHRwczovL3Rvcy5vcmcvb2NlYW5vZ3JhcGh5L2Fzc2V0cy9kb2NzLzMwLTFfYmVyZGFsZXQzLnBkZiIsImlzc3VlZERhdGUiOiIyMDE3LTAzLTAxIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiT2NlYW5vZ3JhcGh5IiwiaXNzdWUiOjEsInZvbHVtZSI6MzAsImltcGFjdEZhY3RvciI6IjIuOTMifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjo3MCwibGFzdFBhZ2UiOjgxfX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6Ik9jZWFub2dyYXBoeSBTb2NpZXR5In0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjoyNiwiY2l0ZWRCeUNvdW50IjozMn19LCJxdW90ZSI6IlRoZSAgZWNvbm9taWMgZWZmZWN0cyBvZiBIQUJzIGFyaXNlIGZyb20gcHViLWxpYyBoZWFsdGggY29zdHMsIGNvbW1lcmNpYWwgZmlzaGVyeSBhbmQgIGFxdWFjdWx0dXJlIG9wZXJhdGlvbiBjbG9zdXJlcyBhbmQgZmlzaCAga2lsbHMsIHBvc3NpYmxlIG1lZGl1bS0gYW5kIGxvbmctdGVybSAgZGVjbGluZXMgaW4gY29hc3RhbCBhbmQgbWFyaW5lIHJlY3JlYXRpb24gIGFuZCB0b3VyaXNtLCBhbmQgdGhlIGNvc3RzIG9mIGluc3VyYW5jZSwgIG1vbml0b3JpbmcsIG1hbmFnZW1lbnQsIGFuZCBtaXRpZ2F0aW9uICAoTW9yZ2FuIGV0IGFsLiwgMjAxMCkuIEFnZ3JlZ2F0aW5nIGVjby1ub21pYyBlZmZlY3Rz4oCUYm90aCB3aXRoaW4gYW5kIGFjcm9zcyAgdGhlc2UgY2F0ZWdvcmllc%2BKAlGNhbiBhbHNvIGJlIHByb2JsZW0tYXRpYyB3aXRoIGF2YWlsYWJsZSBkYXRhIGFuZCBjdXJyZW50bHkgdXNlZCAgbWV0aG9kb2xvZ2llcyAoSG9hZ2xhbmQgZXQgYWwuLCAyMDAyKS4gIEFuIGV4YW1wbGUgb2YgYW4gZWNvbm9taWMgZXZhbHVhdGlvbiAgb2YgSEFCcyBhdCBhIG5hdGlvbmFsIGxldmVsIGlzIGdpdmVuIGJ5ICBBbmRlcnNvbiBldCBhbC4gKDIwMDApLCB3aG8gZXN0aW1hdGVkIHRoZSAgZWNvbm9taWMgZWZmZWN0cyBvZiBIQUJzIGluIHRoZSBVbml0ZWQgIFN0YXRlcyB0byBiZSAkMTAwIG1pbGxpb24gcGVyIHllYXIgKGFzIGxhdGVyICB0cmFuc2xhdGVkIGludG8gMjAxMiBkb2xsYXJzKSwgYnJva2VuIGRvd24gIGFzIGZvbGxvd3M6IDQ1JSBmb3IgcHVibGljIGhlYWx0aCBjb3N0cywgIDM3JSBmb3IgdGhlIGNvc3RzIG9mIGNsb3N1cmVzIGFuZCBsb3NzZXMgIGV4cGVyaWVuY2VkIGJ5IGNvbW1lcmNpYWwgZmlzaGVyaWVzLCAgMTMlIGZyb20gdGhlIGltcGFjdCBvbiBsb3N0IHJlY3JlYXRpb24gIGFuZCB0b3VyaXNtIG9wcG9ydHVuaXRpZXMsIGFuZCA0JSBmcm9tIEZJR1VSRSA2LiBNYXNzaXZlIGZpc2ggbW9ydGFsaXRpZXMgaW4gSmFrYXJ0YSwgSW5kb25lc2lhLiBQaG90byBjcmVkaXRzOiBUdW1wYWsgU2lkYWJ1dGFyIChIQU41MykgbW9uaXRvcmluZyBhbmQgbWFuYWdlbWVudCBjb3N0cy4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjoxMS44MTk0NDQ2NTYzNzIwNywibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzQ5NDUyOTcyNDEyMSwiaGVpZ2h0IjoxLjcwMzI4MjU5NDY4MDc4NjF9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjoxMy41MjM5ODk2Nzc0MjkyLCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI2Ljg2NDM3NDE2MDc2NjYsImhlaWdodCI6MS43MDMyODI1OTQ2ODA3ODYxfX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6MTUuMjI4NTM1NjUyMTYwNjQ1LCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzMjAwMjI1ODMwMDc4LCJoZWlnaHQiOjEuNzAzMjgzNTQ4MzU1MTAyNX19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjE2LjkzMzA4MDY3MzIxNzc3MywibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzM2MzY4NTYwNzkxLCJoZWlnaHQiOjEuNzAzMjgzNTQ4MzU1MTAyNX19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjE4LjYzNzYyNjY0Nzk0OTIyLCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzNTI5NDM0MjA0MTAxNiwiaGVpZ2h0IjoxLjcwMzI4MzU0ODM1NTEwMjV9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjoyMC4zNDIxNzA3MTUzMzIwMywibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzM5ODU5MDA4Nzg5MDYsImhlaWdodCI6MS43MDMyODM1NDgzNTUxMDI1fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6MjIuMDQ2NzE2NjkwMDYzNDc3LCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzMjAxMTc5NTA0Mzk0NSwiaGVpZ2h0IjoxLjcwMzI4MzU0ODM1NTEwMjV9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjoyMy43NTEyNjI2NjQ3OTQ5MjIsImxlZnQiOjY3LjI1NDg5ODA3MTI4OTA2LCJ3aWR0aCI6MjcuMjMyMDQyMzEyNjIyMDcsImhlaWdodCI6MS43MDMyODM1NDgzNTUxMDI1fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6MjUuNDU1ODA4NjM5NTI2MzY3LCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI2Ljg2MzA2NzYyNjk1MzEyNSwiaGVpZ2h0IjoxLjcwMzI4MzU0ODM1NTEwMjV9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjoyNy4xNjAzNTI3MDY5MDkxOCwibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzU0MzM1Nzg0OTEyMSwiaGVpZ2h0IjoxLjcwMzI4MzU0ODM1NTEwMjV9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjoyOC44NjQ4OTg2ODE2NDA2MjUsImxlZnQiOjY3LjI1NDg5ODA3MTI4OTA2LCJ3aWR0aCI6MjYuODY0Mzk1MTQxNjAxNTYyLCJoZWlnaHQiOjEuNzAzMjgzNTQ4MzU1MTAyNX19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjMwLjU2OTQ0Mjc0OTAyMzQzOCwibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzQ5MjQzMTY0MDYyNSwiaGVpZ2h0IjoxLjcwMzI4MzU0ODM1NTEwMjV9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjozMi4yNzM5OTA2MzExMDM1MTYsImxlZnQiOjY3LjI1NDg5ODA3MTI4OTA2LCJ3aWR0aCI6MjcuMjM0Nzc1NTQzMjEyODksImhlaWdodCI6MS43MDMyODM1NDgzNTUxMDI1fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6MzMuOTc4NTM0Njk4NDg2MzMsImxlZnQiOjY3LjI1NDg5ODA3MTI4OTA2LCJ3aWR0aCI6MjcuMjM0MzM2ODUzMDI3MzQ0LCJoZWlnaHQiOjEuNzAzMjg1NDU1NzAzNzM1NH19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjM1LjY4MzA3ODc2NTg2OTE0LCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzMjE2MDU2ODIzNzMwNSwiaGVpZ2h0IjoxLjcwMzI4NTQ1NTcwMzczNTR9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjozNy4zODc2MjI4MzMyNTE5NSwibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzM2NjU0NjYzMDg1OTQsImhlaWdodCI6MS43MDMyODU0NTU3MDM3MzU0fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6MzkuMDkyMTcwNzE1MzMyMDMsImxlZnQiOjY3LjI1NDg5ODA3MTI4OTA2LCJ3aWR0aCI6MjcuMjM2NTc5ODk1MDE5NTMsImhlaWdodCI6MS43MDMyODU0NTU3MDM3MzU0fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6NDAuNzk2NzE0NzgyNzE0ODQ0LCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzNjcxNzIyNDEyMTA5NCwiaGVpZ2h0IjoxLjcwMzI4NTQ1NTcwMzczNTR9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo0Mi41MDEyNjI2NjQ3OTQ5MiwibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzM2MjczMTkzMzU5MzgsImhlaWdodCI6MS43MDMyODU0NTU3MDM3MzU0fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6NDQuMjA1ODA2NzMyMTc3NzM0LCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzMDU0NTA0Mzk0NTMxMiwiaGVpZ2h0IjoxLjcwMzI4NTQ1NTcwMzczNTR9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo0NS45MTAzNTA3OTk1NjA1NSwibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzg1NDQ0NjQxMTEzMjgsImhlaWdodCI6MS43MDMyODU0NTU3MDM3MzU0fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6NDcuNjE0ODk0ODY2OTQzMzYsImxlZnQiOjY3LjI1NDg5ODA3MTI4OTA2LCJ3aWR0aCI6MjcuMjM0MTQ2MTE4MTY0MDYyLCJoZWlnaHQiOjEuNzAzMjg1NDU1NzAzNzM1NH19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjQ5LjMxOTQ0Mjc0OTAyMzQ0LCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzNTk0MDkzMzIyNzU0LCJoZWlnaHQiOjEuNzAzMjg1NDU1NzAzNzM1NH19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjUxLjAyMzk5MDYzMTEwMzUxNiwibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzY5MDc5NTg5ODQzNzUsImhlaWdodCI6MS43MDMyODU0NTU3MDM3MzU0fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6OTIuMjc4NzcwNDQ2Nzc3MzQsImxlZnQiOjguODIzNTI5MjQzNDY5MjM4LCJ3aWR0aCI6NTYuMjYzOTE2MDE1NjI1LCJoZWlnaHQiOjEuMjA5MDg5NTE3NTkzMzgzOH19LHsicGFnZU51bWJlciI6OSwiYmJveCI6eyJ0b3AiOjUuMDAxMjYyNjY0Nzk0OTIyLCJsZWZ0Ijo1Ljg4MjM1MjgyODk3OTQ5Miwid2lkdGgiOjI3LjIzNTI5NDM0MjA0MTAxNiwiaGVpZ2h0IjoxLjcwMzI4MjU5NDY4MDc4NjF9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI3OCJ9fV0%3D. Real estate devaluation near degraded lakes adds hidden costs (Devlin et al., 2020; Sampat et al., 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjEsInN1YnR5cGUiOnt9LCJjb250cmlidXRvcnMiOlt7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBZGFtIFQuIiwiZmFtaWx5IjoiRGV2bGluIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkppYXlpIiwiZmFtaWx5IjoiUGFuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik0uIE1hbmp1ciIsImZhbWlseSI6IlNoYWgifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQWxpeXUiLCJmYW1pbHkiOiJOdWh1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Iktlbm5ldGggVy4iLCJmYW1pbHkiOiJDdW1taW5zIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkNsYXVkaW8gVmFsZG92aW5vcyIsImZhbWlseSI6IlphcmdlcyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQYWJsbyIsImZhbWlseSI6IkZpZXJybyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJWaXZpYW5hIiwiZmFtaWx5IjoiT2xtb3MifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQWJkdWwtUmFobWFuIiwiZmFtaWx5IjoiRGlyaXN1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkpvaG4gT3ZpZSIsImZhbWlseSI6Ik9sb211a29ybyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJOYXRpdmlkYWRlIiwiZmFtaWx5IjoiVmllaXJhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFuYSIsImZhbWlseSI6IlNvYXJlcyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJDYXJsb3MiLCJmYW1pbHkiOiJQaW5oZWlybyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNYXR0aGV3IEYuIiwiZmFtaWx5IjoiQmluZ2hhbSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJKYXNvbiIsImZhbWlseSI6Iktpbm5lbGwifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiV3UiLCJmYW1pbHkiOiJCbyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJKaW5tdSIsImZhbWlseSI6IlpoYW5nIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllpbmRpIiwiZmFtaWx5IjoiWmhhbyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJIdW1iZXJ0byBBbHZlcyIsImZhbWlseSI6IkJhcmJvc2EifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiRnJhbmtsaW4iLCJmYW1pbHkiOiJQYXJlZGVz4oCQVHJlam8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiVCBMYWtzaG1pIiwiZmFtaWx5IjoiS3VtYXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiS2lyYW4iLCJmYW1pbHkiOiJUaGFrdXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQ2F0YXJpbmEiLCJmYW1pbHkiOiJCdXJpdGkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2NvdHQgQS4iLCJmYW1pbHkiOiJXZWxscyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJPc2NhciBPbW9uZGkiLCJmYW1pbHkiOiJEb25kZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJDYXJlbiBOYXZhbGlhIiwiZmFtaWx5IjoiQXRhbGl0c2EifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiRC4iLCJmYW1pbHkiOiJTYW5pIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNhbXBzb24iLCJmYW1pbHkiOiJBaG1lZCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBbWFuaW5nIiwiZmFtaWx5IjoiS3dha3UifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiR2V2b3JnIiwiZmFtaWx5IjoiQWRqZWkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2F0YXJ1cGEiLCJmYW1pbHkiOiJTaW1vbnlhbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTbmlnZGhhIiwiZmFtaWx5IjoiR2hvc2gifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUHJhc2FubmEiLCJmYW1pbHkiOiJDaGF0dGVyamVlIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNoaXZhIiwiZmFtaWx5IjoiR2hvcmEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQmVsZGEiLCJmYW1pbHkiOiJQcmFzYWQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiS2V0aGxhdGxvZ2lsZSIsImZhbWlseSI6Ik1vc2VwZWxlIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ill1c3VmIiwiZmFtaWx5IjoiU3VsZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJVaXLDoSIsImZhbWlseSI6Ik9saXZlaXJhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1hcmlhIEpvw6NvIiwiZmFtaWx5IjoiVmllaXJhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkZsZXVyIiwiZmFtaWx5IjoiRGUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTWluIiwiZmFtaWx5IjoiTWF4In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikdldm9yZyIsImZhbWlseSI6IlNpbW9ueWFuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNhdGFydXBhIiwiZmFtaWx5IjoiR2hvc2gifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU25pZ2RoYSIsImZhbWlseSI6IkNoYXR0ZXJqZWUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUHJhc2FubmEiLCJmYW1pbHkiOiJQcmFzYWQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2F0YXJ1cGEiLCJmYW1pbHkiOiJHaG9zaCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJHaG9yYSIsImZhbWlseSI6IlByYXNhZCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQcmFzYW5uYSIsImZhbWlseSI6IlBhbCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLd2FrdSIsImZhbWlseSI6IkFnb2R6byJ9fV0sImlkcyI6eyJpc3NuIjpbXSwiaXNibiI6W10sImRvaSI6IjEwLjU3NzIvaW50ZWNob3Blbi44NzQ2MyIsIm1hZyI6IjMxMjg1ODc0NjkiLCJvcGVuYWxleCI6IlczMTI4NTg3NDY5In0sInRpdGxlIjoiSW5sYW5kIFdhdGVycyAtIER5bmFtaWNzIGFuZCBFY29sb2d5IiwiYWJzdHJhY3QiOiJJbmxhbmQgd2F0ZXJzLCBsYWtlcywgcml2ZXJzLCBhbmQgdGhlaXIgY29ubmVjdGVkIHdldGxhbmRzIGFyZSB0aGUgbW9zdCBpbXBvcnRhbnQgYW5kIHRoZSBtb3N0IHZ1bG5lcmFibGUgc291cmNlcyBvZiBmcmVzaHdhdGVyIG9uIHRoZSBwbGFuZXQuIFRoZSBlY29sb2d5IG9mIHRoZXNlIHN5c3RlbXMgaW5jbHVkZXMgYmlvbG9neSBhcyB3ZWxsIGFzIGh1bWFuIHBvcHVsYXRpb25zIGFuZCBjaXZpbGl6YXRpb24uIElubGFuZCB3YXRlcnMgYW5kIHdldGxhbmRzIGFyZSBoaWdobHkgc3VzY2VwdGlibGUgdG8gY2hlbWljYWwgYW5kIGJpb2xvZ2ljYWwgcG9sbHV0YW50cyBmcm9tIG5hdHVyYWwgb3IgaHVtYW4gc291cmNlcywgY2hhbmdlcyBpbiB3YXRlcnNoZWQgZHluYW1pY3MgZHVlIHRvIHRoZSBlc3RhYmxpc2htZW50IG9mIGRhbXMgYW5kIHJlc2Vydm9pcnMsIGFuZCBsYW5kIHVzZSBjaGFuZ2VzIGZyb20gYWdyaWN1bHR1cmUgYW5kIGluZHVzdHJ5LiBUaGlzIGJvb2sgcHJvdmlkZXMgYSBjb21wcmVoZW5zaXZlIHJldmlldyBvZiBpc3N1ZXMgaW52b2x2aW5nIGlubGFuZCB3YXRlcnMgYW5kIGRpc2N1c3NlcyBtYW55IHdvcmxkd2lkZSBpbmxhbmQgd2F0ZXIgc3lzdGVtcy4gVGhlIG1haW4gdG9waWNzIG9mIHRoaXMgdGV4dCBhcmUgd2F0ZXIgcXVhbGl0eSBpbnZlc3RpZ2F0aW9uLCBhbmFseXNlcyBvZiB0aGUgZWNvbG9neSBvZiBpbmxhbmQgd2F0ZXIgc3lzdGVtcywgcmVtb3RlIHNlbnNpbmcgb2JzZXJ2YXRpb24gYW5kIG51bWVyaWNhbCBtb2RlbGluZyBtZXRob2RzLCBhbmQgYmlvZGl2ZXJzaXR5IGludmVzdGlnYXRpb25zLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC41NzcyL2ludGVjaG9wZW4uODc0NjMiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vbXRzLmludGVjaG9wZW4uY29tL3N0b3JhZ2UvYm9va3MvOTY2MC9hdXRob3JzX2Jvb2svYXV0aG9yc19ib29rLnBkZiIsImlzc3VlZERhdGUiOiIyMDIwLTA0LTAxIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiSW50ZWNoT3BlbiBlQm9va3MiLCJpbXBhY3RGYWN0b3IiOiIxLjAxIn0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkludGVjaE9wZW4ifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjMzOCwiY2l0ZWRCeUNvdW50IjoyN319LCJxdW90ZSI6IkFkZGl0aW9uYWxseSwgc2luY2UgYWZmZWN0ZWQgYnVzaW5lc3Nlc%2BKAmSB2YWx1ZXMgIGFyZSBtb3N0IGxpa2VseSB0aWVkIHRvIHRoZWlyIGFzc2V0cyBhbmQgdGhlIHJlYWwgZXN0YXRlIHRoZXkgb2NjdXB5IChmb3IgZXhhbXBsZSwgIGEgbWFyaW5hIGlzIG5vdCBlYXNpbHkgY29udmVydGVkIHRvIHNvbWUgb3RoZXIgdXNlKSwgb24tZ29pbmcgYmFsYW5jZSBzaGVldCBlZmZlY3RzICB3b3VsZCB1bHRpbWF0ZWx5IGxlYWQgdG8gcmVkdWN0aW9ucyBpbiBjb21tZXJjaWFsIHJlYWwgZXN0YXRlIHZhbHVlcy4gVGhlcmUgYXJlIG1hbnkgY2hhbGxlbmdlcyB0byB1bmRlcnN0YW5kaW5nIHRoZSBpbXBsaWNhdGlvbnMgb2YgY2hhbmdlcyBpbiAgdG91cmlzbSBmcm9tIEhBQnMuIFRoZSBjbGVhcmVzdCBjaGFsbGVuZ2Ugb2JzdHJ1Y3RpbmcgYSBwcmVjaXNlIGFuYWx5c2lzIG9mIHRoZXNlICBpbXBhY3RzIGlzIGEgbGFjayBvZiBkYXRhIGVpdGhlciBvbiB0aGUgYW1vdW50IG9mIHRvdXJpc20gYXQgcmlzayBvciB0aGUgc3BlY2lmaWMgIGltcGFjdCBvZiBIQUJzIG9uIHRvdXJpc20uIEZvciBleGFtcGxlLCB3aGlsZSBjb3VudHktbGV2ZWwgZGF0YSBleGlzdHMgZm9yIHRvdGFsICBleHBlbmRpdHVyZXMgb24gdG91cmlzbSwgdGhpcyBpbmNsdWRlcyB0b3VyaXNtIHdoaWNoIHdvdWxkIG5vdCBiZSBpbnRlcnJ1cHRlZCBieSAgSEFCcyBvciBvdGhlciBkaXNjb3VyYWdpbmcgZmFjdG9ycy4gQW4gYWRkaXRpb25hbCBjaGFsbGVuZ2UgcmVsYXRlcyB0byB0aGUgZGlzdGluY3Rpb24gYmV0d2VlbiBlY29ub21pYyBiZW5lZml0cyAgKHdpbGxpbmduZXNzIHRvIHBheSkgYW5kIGVjb25vbWljIGltcGFjdHMgKGV4cGVuZGl0dXJlcyksIGFuZCB0aGUgbWVhc3VyZS1tZW50IG9mIHRoZSBlY29ub21pYyBiZW5lZml0cyB0aGF0IGFyaXNlIGZyb20gZWNvbm9taWMgaW1wYWN0cyAocHJvZml0cykuIEZvciAgZXhhbXBsZSwgY29uc2lkZXIgYSByZXN0YXVyYW50IG93bmVyIHdobyBsb3NlcyAkMTAsMDAwIGluIHJldmVudWUgYmVjYXVzZSBvZiBhICBIQUIuIFRoZSBvd25lcuKAmXMgd2lsbGluZ25lc3MgdG8gcGF5IHRvIHJlY292ZXIgdGhhdCByZXZlbnVlOyBpcyAocm91Z2hseSBzcGVha2luZykgIHRoZSBsb3N0IHByb2ZpdCBvbiB0aGF0IHJldmVudWUuIFRoaXMgaXMgbW9yZSBkaWZmaWN1bHQgdG8gaWRlbnRpZnkgdGhhbiBsb3N0IHJldmVudWUuICAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6ODksImJib3giOnsidG9wIjo1OS40NDI4NjcyNzkwNTI3MzQsImxlZnQiOjE0LjcyMjg1MzY2MDU4MzQ5Niwid2lkdGgiOjY4Ljc0ODAwMTA5ODYzMjgxLCJoZWlnaHQiOjEuOTA1MDAwODA1ODU0Nzk3NH19LHsicGFnZU51bWJlciI6ODksImJib3giOnsidG9wIjo2MS4wNzEwNzU0Mzk0NTMxMjUsImxlZnQiOjE0LjcyMjg1MzY2MDU4MzQ5Niwid2lkdGgiOjY2LjM5NjM0NzA0NTg5ODQ0LCJoZWlnaHQiOjEuOTA1MDAwODA1ODU0Nzk3NH19LHsicGFnZU51bWJlciI6ODksImJib3giOnsidG9wIjo2Mi42OTkyNzU5NzA0NTg5ODQsImxlZnQiOjE0LjcyMjg1MzY2MDU4MzQ5Niwid2lkdGgiOjY5LjI2OTMwOTk5NzU1ODYsImhlaWdodCI6MS45MDUwMDA4MDU4NTQ3OTc0fX0seyJwYWdlTnVtYmVyIjo4OSwiYmJveCI6eyJ0b3AiOjY0LjMyNzQ4NDEzMDg1OTM4LCJsZWZ0IjoxNC43MjI4NTM2NjA1ODM0OTYsIndpZHRoIjo1Ny40MDA1MDUwNjU5MTc5NywiaGVpZ2h0IjoxLjkwNTAwMDgwNTg1NDc5NzR9fSx7InBhZ2VOdW1iZXIiOjg5LCJiYm94Ijp7InRvcCI6NjUuOTU1Njg4NDc2NTYyNSwibGVmdCI6MTcuNSwid2lkdGgiOjYzLjA5NTY5OTMxMDMwMjczNCwiaGVpZ2h0IjoxLjkwNTAwMDgwNTg1NDc5NzR9fSx7InBhZ2VOdW1iZXIiOjg5LCJiYm94Ijp7InRvcCI6NjcuNTgzODkyODIyMjY1NjIsImxlZnQiOjE0LjcyMjg1MzY2MDU4MzQ5Niwid2lkdGgiOjY4LjUyMjU5ODI2NjYwMTU2LCJoZWlnaHQiOjEuOTA0OTk2NjMzNTI5NjYzfX0seyJwYWdlTnVtYmVyIjo4OSwiYmJveCI6eyJ0b3AiOjY5LjIxMjA4OTUzODU3NDIyLCJsZWZ0IjoxNC43MjI4NTM2NjA1ODM0OTYsIndpZHRoIjo2NS40MTIyNzcyMjE2Nzk2OSwiaGVpZ2h0IjoxLjkwNTAwMDgwNTg1NDc5NzR9fSx7InBhZ2VOdW1iZXIiOjg5LCJiYm94Ijp7InRvcCI6NzAuODQwMjkzODg0Mjc3MzQsImxlZnQiOjE0LjcyMjg1MzY2MDU4MzQ5Niwid2lkdGgiOjY3Ljc3MDAxOTUzMTI1LCJoZWlnaHQiOjEuOTA1MDA1MDk3Mzg5MjIxMn19LHsicGFnZU51bWJlciI6ODksImJib3giOnsidG9wIjo3Mi40Njg0OTgyMjk5ODA0NywibGVmdCI6MTQuNzIyODUzNjYwNTgzNDk2LCJ3aWR0aCI6NjkuMzg2MzI5NjUwODc4OSwiaGVpZ2h0IjoxLjkwNTAwNTA5NzM4OTIyMTJ9fSx7InBhZ2VOdW1iZXIiOjg5LCJiYm94Ijp7InRvcCI6NzQuMDk2NzAyNTc1NjgzNiwibGVmdCI6MTQuNzIyODUzNjYwNTgzNDk2LCJ3aWR0aCI6MzAuMTM0ODgwMDY1OTE3OTcsImhlaWdodCI6MS45MDUwMDUwOTczODkyMjEyfX0seyJwYWdlTnVtYmVyIjo4OSwiYmJveCI6eyJ0b3AiOjc1LjcyNDkxNDU1MDc4MTI1LCJsZWZ0IjoxNy41LCJ3aWR0aCI6NjQuNDEwNzk3MTE5MTQwNjIsImhlaWdodCI6MS45MDUwMDUwOTczODkyMjEyfX0seyJwYWdlTnVtYmVyIjo4OSwiYmJveCI6eyJ0b3AiOjc3LjM1MzExMTI2NzA4OTg0LCJsZWZ0IjoxNC43MjI4NTM2NjA1ODM0OTYsIndpZHRoIjo2NC44NTU2OTAwMDI0NDE0LCJoZWlnaHQiOjEuOTA1MDA1MDk3Mzg5MjIxMn19LHsicGFnZU51bWJlciI6ODksImJib3giOnsidG9wIjo3OC45ODEzMTU2MTI3OTI5NywibGVmdCI6MTQuNzIyODUzNjYwNTgzNDk2LCJ3aWR0aCI6NjYuMDE5ODUxNjg0NTcwMzEsImhlaWdodCI6MS45MDUwMDUwOTczODkyMjEyfX0seyJwYWdlTnVtYmVyIjo4OSwiYmJveCI6eyJ0b3AiOjgwLjYwOTUyNzU4Nzg5MDYyLCJsZWZ0IjoxNC43MjI4NTM2NjA1ODM0OTYsIndpZHRoIjo2Ny43ODc2NDM0MzI2MTcxOSwiaGVpZ2h0IjoxLjkwNTAwNTA5NzM4OTIyMTJ9fSx7InBhZ2VOdW1iZXIiOjg5LCJiYm94Ijp7InRvcCI6ODIuMjM3NzI0MzA0MTk5MjIsImxlZnQiOjE0LjcyMjg1MzY2MDU4MzQ5Niwid2lkdGgiOjY5LjczOTY5MjY4Nzk4ODI4LCJoZWlnaHQiOjEuOTA1MDA1MDk3Mzg5MjIxMn19LHsicGFnZU51bWJlciI6ODksImJib3giOnsidG9wIjo4My44NjU5MzYyNzkyOTY4OCwibGVmdCI6MTQuNzIyODUzNjYwNTgzNDk2LCJ3aWR0aCI6NjguNDg3MTI5MjExNDI1NzgsImhlaWdodCI6MS45MDUwMDUwOTczODkyMjEyfX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiOTAifX0seyJvcmlnaW4iOjUsIndvcmsiOnsidHlwZSI6OCwic3VidHlwZSI6eyJvbmVvZktpbmQiOiJhcnRpY2xlVHlwZSIsImFydGljbGVUeXBlIjoxfSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQXBvb3J2YSBNLiIsImZhbWlseSI6IlNhbXBhdCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBbmRyZWEiLCJmYW1pbHkiOiJIaWNrcyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJHZXJhcmRvIEouIiwiZmFtaWx5IjoiUnVpeuKAkE1lcmNhZG8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiVsOtwq1jdG9yIE0uIiwiZmFtaWx5IjoiWmF2YWxhIn19XSwiaWRzIjp7Imlzc24iOlsiMDkyMS0zNDQ5IiwiMTg3OS0wNjU4Il0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDE2L2oucmVzY29ucmVjLjIwMjAuMTA1MTk5IiwibWFnIjoiMzA5NDYyNTEwNCIsInBtaWQiOiIzMzc0NjM2MSIsIm9wZW5hbGV4IjoiVzMwOTQ2MjUxMDQifSwidGl0bGUiOiJWYWx1aW5nIGVjb25vbWljIGltcGFjdCByZWR1Y3Rpb25zIG9mIG51dHJpZW50IHBvbGx1dGlvbiBmcm9tIGxpdmVzdG9jayB3YXN0ZSIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2oucmVzY29ucmVjLjIwMjAuMTA1MTk5IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5uY2JpLm5sbS5uaWguZ292L3BtYy9hcnRpY2xlcy83OTcwNTA1IiwiaXNzdWVkRGF0ZSI6IjIwMjAtMTAtMTUiLCJhY2Nlc3NlZERhdGUiOiIyMDI2LTAxIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJSZXNvdXJjZXMgQ29uc2VydmF0aW9uIGFuZCBSZWN5Y2xpbmciLCJ2b2x1bWUiOjE2NCwiaW1wYWN0RmFjdG9yIjoiMTEuMDEifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjoxMDUxOTksImxhc3RQYWdlIjoxMDUxOTl9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRWxzZXZpZXIgQlYifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjM5LCJjaXRlZEJ5Q291bnQiOjU3fX0sInF1b3RlIjoiU2VjdGlvbiBUaXRsZTogVmFsdWluZyBlY29ub21pYyBpbXBhY3QgcmVkdWN0aW9ucyBvZiBudXRyaWVudCBwb2xsdXRpb24gZnJvbSBsaXZlc3RvY2sgd2FzdGUgPiAxLiBJbnRyb2R1Y3Rpb25cbkNvbnRlbnQ6XG5JbiB0aGUgU3RhdGUgb2YgRmxvcmlkYSwgSEFCcyBoYXZlIHJlc3VsdGVkIGluIGEgbW9udGhseSBsb3NzIG9mIDIuOCBhbmQgMy43IG1pbGxpb24gVVNEIGNvcnJlc3BvbmRpbmcgdG8gcmVzdGF1cmFudCBhbmQgbG9kZ2luZyByZXZlbnVlLCByZXNwZWN0aXZlbHkgKCBbTGFya2luIGFuZCBBZGFtcywgMjAwN10oKSApLiBGcmVxdWVudCBvY2N1cnJlbmNlIG9mIEhBQnMgYWxzbyBsb3dlcnMgcHJvcGVydHkgdmFsdWVzIG9mIGxha2Vmcm9udCBwcm9wZXJ0aWVzLiBUaGUgbG9zcyBpbiBwcm9wZXJ0eSB2YWx1ZXMgYXJlIHRoZSBsYXJnZXN0IGltcGFjdCBiZWFyZXJzIG9mIGV1dHJvcGhpY2F0aW9uIHdpdGggYW4gZXN0aW1hdGVkIGF2ZXJhZ2UgZWNvbm9taWMgbG9zcyBvZiAxLjYgYmlsbGlvbiBVU0QgYW5udWFsbHkgKCBbRG9kZHMgZXQgYWwuLCAyMDA5XSgpICkuIn1d. Urgent, decentralized interventions buffers, monitoring are vital for resilience. However, the efficacy of these measures is frequently constrained by the "wicked" nature of eutrophication, which arises from cumulative processes and lacks broadly applicable best management practices due to the unique hydrological and socio-economic characteristics of each watershed (Khorasani & Zhu, 2021)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhhbWVkIiwiZmFtaWx5IjoiS2hvcmFzYW5pIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlpoZW5kdW8iLCJmYW1pbHkiOiJaaHUifX1dLCJpZHMiOnsiaXNzbiI6WyIwMDIyLTE2OTQiLCIxODc5LTI3MDciXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMTYvai5qaHlkcm9sLjIwMjEuMTI2ODg2IiwibWFnIjoiMzE3NzA3MTI5OCIsIm9wZW5hbGV4IjoiVzMxNzcwNzEyOTgiLCJvcGVuQWNjZXNzSWQiOiI5ODcxNjM2Mjg2OTgwMDIwOTMifSwidGl0bGUiOiJQaG9zcGhvcnVzIHJldGVudGlvbiBpbiBsYWtlczogQSBjcml0aWNhbCByZWFzc2Vzc21lbnQgb2YgaHlwb3RoZXNlcyBhbmQgc3RhdGljIG1vZGVscyIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2ouamh5ZHJvbC4yMDIxLjEyNjg4NiIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9lYXJ0aGFyeGl2Lm9yZy9yZXBvc2l0b3J5L29iamVjdC8yNDgxL2Rvd25sb2FkLzUwNzQvIiwiaXNzdWVkRGF0ZSI6IjIwMjEtMDktMDIiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA4IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJKb3VybmFsIG9mIEh5ZHJvbG9neSIsInZvbHVtZSI6NjAzLCJpbXBhY3RGYWN0b3IiOiI3LjY0In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTI2ODg2LCJsYXN0UGFnZSI6MTI2ODg2fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkVsc2V2aWVyIEJWIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo3NywiY2l0ZWRCeUNvdW50IjoxMn19LCJxdW90ZSI6Ikhvd2V2ZXIsIGh1bWFuIGFjdGl2aXRpZXMgaGF2ZSA0OSBhY2NlbGVyYXRlZCB0aGUgZXV0cm9waGljYXRpb24gcHJvY2VzcyBieSBpbmNyZWFzaW5nIHRoZSBudXRyaWVudHMgZGVsaXZlcnkgdG8gdGhlIGFxdWF0aWMgc3lzdGVtcyA1MCAoTWVrb25uZW4gYW5kIEhvZWtzdHJhLCAyMDE4KS4gVGh1cywgYW50aHJvcG9nZW5pYyBldXRyb3BoaWNhdGlvbiBpcyBvbmUgb2YgdGhlIG1vc3QgaW1wb3J0YW50IDUxIGVsZW1lbnRzIG9mIGZyZXNoIGFuZCBtYXJpbmUgd2F0ZXIgcXVhbGl0eSBkZXRlcmlvcmF0aW9uIChIdSBldCBhbC4sIDIwMTk7IFNtaXRoIGFuZCBTY2hpbmRsZXIsIDUyIDIwMDkpLiBPbmUgZGlyZWN0IGNvbnNlcXVlbmNlIG9mIGFudGhyb3BvZ2VuaWMgZXV0cm9waGljYXRpb24gY29tZXMgaW4gdGhlIGZvcm0gb2YgbWFzc2l2ZSBhbGdhbCA1MyBibG9vbXMgKEdyYW7DqWxpIGV0IGFsLiwgMjAwODsgSGVpc2xlciBldCBhbC4sIDIwMDgpLCB3aGljaCBhcmUgcHJlZGljdGVkIHRvIGJlIGludGVuc2lmaWVkIHVuZGVyIDU0IHdhcm1lciB3YXRlciB0ZW1wZXJhdHVyZXMgYXMgY2xpbWF0ZSBjaGFuZ2VzIChHb2JsZXIsIDIwMjA7IE11a3VuZGFuIGV0IGFsLiwgMjAyMCkuIDU1IEV1dHJvcGhpY2F0aW9uIGlzIGEg4oCcd2lja2Vk4oCdIHByb2JsZW0sIHdoaWNoIGlzIHRoZSBjb25zZXF1ZW5jZSBvZiB2YXJpb3VzIHByb2Nlc3NlcyB0aGF0IG9wZXJhdGUgNTYgY3VtdWxhdGl2ZWx5LiBDb25zaWRlcmluZyB0aGUgdW5pcXVlbmVzcyBvZiBlYWNoIGxha2UgYW5kIGl0cyBzdXJyb3VuZGluZyBhcmVhLCB0aGVyZSBpcyBubyBicm9hZGx5IDU3IGFwcGxpY2FibGUgc2V0IG9mIGJlc3QgbWFuYWdlbWVudCBwcmFjdGljZXMgdGhhdCBjYW4gYmUgYXBwbGllZCBpbiB3YXRlcnNoZWRzIHRvIG1pdGlnYXRlIDU4IHBob3NwaG9ydXMgbG9hZGluZyBhbmQgaXRzIGltcGFjdCBvbiBhbGwgbGFrZXMgKFRob3JudG9uIGV0IGFsLiwgMjAxMykuIEhlbmNlLCBldXRyb3BoaWNhdGlvbiA1OSBtYW5hZ2VtZW50IGFuZCBsYWtlIHJlc3RvcmF0aW9uIG5lZWQgaW50ZWdyYXRlZCBwbGFucyB0aGF0IGFyZSBub3Qgb25seSBzY2llbnRpZmljYWxseSB2YWxpZCBidXQgNjAgYWxzbyBzb2Npby1lY29ub21pY2FsbHkgc2F0aXNmeWluZyAoR2lic29uIGV0IGFsLiwgMjAwMCkuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MzYuMTY4NDM0MTQzMDY2NDA2LCJsZWZ0IjoxMS43Njg2MjgxMjA0MjIzNjMsIndpZHRoIjo3Ni45NjMzOTQxNjUwMzkwNiwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjM2LjcxODQzMzM4MDEyNjk1LCJsZWZ0Ijo2Ljk4MDM5MjQ1NjA1NDY4NzUsIndpZHRoIjoyLjI1MDk1NzQ4OTAxMzY3MiwiaGVpZ2h0IjoxLjM5Mzk0MDQ0ODc2MDk4NjN9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjozOS42NTMyODU5ODAyMjQ2MSwibGVmdCI6Ni45ODAzOTI0NTYwNTQ2ODc1LCJ3aWR0aCI6ODEuNzYxNDI4ODMzMDA3ODEsImhlaWdodCI6Mi4wMjEyMTE2MjQxNDU1MDh9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo0My4xMzgxMzQwMDI2ODU1NSwibGVmdCI6Ni45ODAzOTI0NTYwNTQ2ODc1LCJ3aWR0aCI6ODEuNzU1NTYxODI4NjEzMjgsImhlaWdodCI6Mi4wMjEyMTE2MjQxNDU1MDh9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo0Ni42MjI5ODIwMjUxNDY0ODQsImxlZnQiOjYuOTgwMzkyNDU2MDU0Njg3NSwid2lkdGgiOjgxLjc0MjgwNTQ4MDk1NzAzLCJoZWlnaHQiOjIuMDIxMjExNjI0MTQ1NTA4fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6NTAuMTA3ODMwMDQ3NjA3NDIsImxlZnQiOjYuOTgwMzkyNDU2MDU0Njg3NSwid2lkdGgiOjgxLjc1MjYxNjg4MjMyNDIyLCJoZWlnaHQiOjIuMDIxMjExNjI0MTQ1NTA4fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6NTMuNTk1MjAzMzk5NjU4MiwibGVmdCI6Ni45ODAzOTI0NTYwNTQ2ODc1LCJ3aWR0aCI6ODEuNzQ4NzAzMDAyOTI5NjksImhlaWdodCI6Mi4wMjEyMTE2MjQxNDU1MDh9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo1Ny4wODAwNDc2MDc0MjE4NzUsImxlZnQiOjYuOTgwMzkyNDU2MDU0Njg3NSwid2lkdGgiOjczLjMxMDQ1NTMyMjI2NTYyLCJoZWlnaHQiOjIuMDIxMjExNjI0MTQ1NTA4fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6NjEuNTY0OTAzMjU5Mjc3MzQ0LCJsZWZ0Ijo2Ljk4MDM5MjQ1NjA1NDY4NzUsIndpZHRoIjo4MS43NTc1MTQ5NTM2MTMyOCwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjY1LjA0OTc0MzY1MjM0Mzc1LCJsZWZ0Ijo2Ljk4MDM5MjQ1NjA1NDY4NzUsIndpZHRoIjo4MS43NTk0NzU3MDgwMDc4MSwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjY4LjUzNDU5MTY3NDgwNDY5LCJsZWZ0Ijo2Ljk4MDM5MjQ1NjA1NDY4NzUsIndpZHRoIjo4MS43MDM5MzM3MTU4MjAzMSwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjcyLjAxOTQ1NDk1NjA1NDY5LCJsZWZ0Ijo2Ljk4MDM5MjQ1NjA1NDY4NzUsIndpZHRoIjo4MS43NTA2NTYxMjc5Mjk2OSwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjc1LjUwODA4NzE1ODIwMzEyLCJsZWZ0Ijo2Ljk4MDM5MjQ1NjA1NDY4NzUsIndpZHRoIjo4MS43NTc1MTQ5NTM2MTMyOCwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjc4Ljk5MjkzNTE4MDY2NDA2LCJsZWZ0IjoxMS43Njg2MjgxMjA0MjIzNjMsIndpZHRoIjo3Ni45NzMxOTc5MzcwMTE3MiwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19XX0sImNodW5rSWQiOiI0NWMwZjRiMy1mM2YxLTRjNTMtYjdmMC1lMmE3MTdjMDlmNTEiLCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIxMjY4ODgifX1d. Furthermore, national water quality standards have frequently proven insufficient to prevent these blooms, as evidenced by the lack of definitive success stories in remediation efforts, which often remain merely management narratives rather than true restoration outcomes (Foulon et al., 2019)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IsOJdGllbm5lIiwiZmFtaWx5IjoiRm91bG9uIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFsYWluIE4uIiwiZmFtaWx5IjoiUm91c3NlYXUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiR2xlbm4iLCJmYW1pbHkiOiJCZW5veSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJSZWJlY2NhIEwuIiwiZmFtaWx5IjoiTm9ydGgifX1dLCJpZHMiOnsiaXNzbiI6WyIxMjAxLTMwODAiLCIyNDA4LTk0NDMiXSwiaXNibiI6W10sImRvaSI6IjEwLjIxNjYvd3Fyai4yMDE5LjAxMyIsIm1hZyI6IjI5NzM4NjAyODEiLCJvcGVuYWxleCI6IlcyOTczODYwMjgxIiwib3BlbkFjY2Vzc0lkIjoiMTY4MDgwMDY5OTc5MTQzNTcwNTcifSwidGl0bGUiOiJBIGdsb2JhbCBzY2FuIG9mIGhvdyB0aGUgaXNzdWUgb2YgbnV0cmllbnQgbG9hZGluZyBhbmQgaGFybWZ1bCBhbGdhbCBibG9vbXMgaXMgYmVpbmcgYWRkcmVzc2VkIGJ5IGdvdmVybm1lbnRzLCBub24tZ292ZXJubWVudGFsIG9yZ2FuaXphdGlvbnMsIGFuZCB2b2x1bnRlZXJzIiwiYWJzdHJhY3QiOiJBYnN0cmFjdCBIYXJtZnVsIGFsZ2FsIGJsb29tcyAoSEFCcykgaW4gYXF1YXRpYyBlY29zeXN0ZW1zIGFyZSBvZiBjb25jZXJuIHdvcmxkd2lkZS4gVGhpcyByZXZpZXcgZGVhbHMgd2l0aCBob3cganVyaXNkaWN0aW9ucyBhcm91bmQgdGhlIHdvcmxkIGFyZSBhZGRyZXNzaW5nIHRoaXMgd2F0ZXIgcXVhbGl0eSBpc3N1ZSB0byBpbmZvcm0gcmVjb21tZW5kYXRpb25zIHJlZ2FyZGluZyBudXRyaWVudCBsb2FkaW5nIGFuZCBIQUJzIGluIE1pc3Npc3F1b2kgQmF5LUxha2UgQ2hhbXBsYWluIGFuZCBMYWtlIE1lbXBocmVtYWdvZzsgdHJhbnNib3VuZGFyeSBsYWtlcyBsb2NhdGVkIGluIHRoZSBVU0EgYW5kIENhbmFkYSB0aGF0IHN1ZmZlciBmcm9tIHN5bXB0b21zIG9mIGV1dHJvcGhpY2F0aW9uLiBBIGdsb2JhbCBzY2FuIG9mIHRoZSBsaXRlcmF0dXJlIHJlc3VsdGVkIGluIHRoZSBjb25zaWRlcmF0aW9uIG9mIDEyIGNhc2Ugc3R1ZGllcyBvZiBsYXJnZSB3YXRlciBib2RpZXMgd2l0aGluIGxhcmdlIHdhdGVyc2hlZHMsIGV4Y2x1ZGluZyBpbi1sYWtlIGdlb2VuZ2luZWVyaW5nIGFwcHJvYWNoZXMuIEFsdGhvdWdoIGFsbCBvZiB0aGUgc3lzdGVtcyBleHBlcmllbmNlIGV4Y2Vzc2l2ZSBudXRyaWVudCBsb2FkaW5nLCB0aGV5IHZhcnkgaW4gdHdvIGtleSB3YXlzOiBzb3VyY2VzIG9mIG51dHJpZW50cyBhbmQgbWFuaWZlc3RhdGlvbnMgb2YgZXV0cm9waGljYXRpb24gcmFuZ2luZyBmcm9tIEhBQnMsIHRvIGxpbWl0ZWQgcmVjcmVhdGlvbmFsIHVzZXMsIHRvIHRoZSBhZGRpdGlvbmFsIGNvbXBsZXhpdHkgb2YgaW50ZXJuYWwgbG9hZGluZ3MgYW5kIGZpc2gga2lsbHMsIHVwIHRvIGRyaW5raW5nIHdhdGVyIHNodXRkb3ducy4gVGhlIGNhc2Ugc3R1ZGllcyB3ZXJlIGFuYWx5emVkIHdpdGggcmVzcGVjdCB0byBmb3VyIGNhdGVnb3JpZXMgb2YgYXBwcm9hY2hlcywgbmFtZWx5OiAoaSkgcmVndWxhdG9yeTsgKGlpKSBpbmNlbnRpdmUtYmFzZWQ7IChpaWkpIHJpc2sgbWl0aWdhdGlvbjsgYW5kIChpdikgb3V0cmVhY2gsIGVuZ2FnZW1lbnQsIGFuZCBlZHVjYXRpb25hbC4gV2UgZm91bmQgdGhhdCB0aGUgbWFuYWdlbWVudCBmcmFtZXdvcmtzIGFyZSBiYXNlZCBvbiBpbnRlZ3JhdGVkIHdhdGVyc2hlZCBtYW5hZ2VtZW50IHBsYW5uaW5nIGFuZCBuYXRpb25hbCBzdGFuZGFyZHMuIE5hdGlvbmFsIHdhdGVyIHF1YWxpdHkgc3RhbmRhcmRzLCBob3dldmVyLCBhcmUgbm90IHN0cmluZ2VudCBlbm91Z2ggdG8gcHJldmVudCBIQUJzLiBPdmVyYWxsLCBpZGVudGlmaWVkIGNhc2Ugc3R1ZGllcyBkaWQgbm90IHN1Y2Nlc3NmdWxseSByZW1lZGlhdGUgSEFCcywgdGhleSBzaW1wbHkgbWFuYWdlZCB0aGVtLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4yMTY2L3dxcmouMjAxOS4wMTMiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vaXdhcG9ubGluZS5jb20vd3Fyai9hcnRpY2xlLXBkZi81NS8xLzEvNzA5MjYxL3dxcmpjMDU1MDAwMS5wZGYiLCJpc3N1ZWREYXRlIjoiMjAxOS0wOS0xNyIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDkiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IldhdGVyIFF1YWxpdHkgUmVzZWFyY2ggSm91cm5hbCIsImlzc3VlIjoxLCJ2b2x1bWUiOjU1LCJpbXBhY3RGYWN0b3IiOiI3LjgzIn0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MSwibGFzdFBhZ2UiOjIzfX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlVXQSBQdWJsaXNoaW5nIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo4NCwiY2l0ZWRCeUNvdW50IjoyMH19LCJxdW90ZSI6IldlIHByb3Bvc2Ugc29sdXRpb25zIGFuZCBwYXRocyBmb3J3YXJkIHRvIGEgd2lja2VkIHByb2JsZW0gd2hpY2ggc2hvdWxkIHByb3ZlIGluc2lnaHRmdWwgZm9yIHBvbGljeS1tYWtlcnMgYW5kIG1hbi0gYWdlcnMgYWxpa2UgdGhyb3VnaCB0aGUgZm9sbG93aW5nIGZpbmRpbmdzLiBOYXRpb25hbCB3YXRlciBxdWFsaXR5IHN0YW5kYXJkcyBoYXZlIG5vdCBwcm92ZWQgc3RyaW5nZW50IGVub3VnaCB0byBwcmUtIHZlbnQgSEFCcy4gSWRlbnRpZmllZCBmcmFtZXdvcmtzIGFyZSBhbGwgYmFzZWQgb24gaW50ZWdyYXRlZCB3YXRlcnNoZWQgbWFuYWdlbWVudCBhbmQgbmF0aW9uYWwgc3RhbmRhcmRzLiBObyBhY3R1YWwgc3VjY2VzcyBzdG9yaWVzIHJlbGF0ZWQgdG8gcmVtZWRpYXRpb24gb2YgSEFCcyB3ZXJlIGlkZW50aWZpZWQ7IHRoZXkgd2VyZSBzaW1wbHkgbWFuYWdlbWVudCBzdG9yaWVzLiBUaGUgZWZmaWNhY3kgb2YgTlBTIEJNUHMgaXMgcXVlc3Rpb25lZCB0aHJvdWdob3V0IHRoZSB3b3JsZCBkdWUgdG8gaW5zdWZmaWNpZW50IGV2aWRlbmNlIHRvIGRldGVybWluZSB3aGV0aGVyIHRoZXkgY2FuIG1pdGlnYXRlIEhBQnMuIE1hcmtldC1iYXNlZCBzdHJhdGVnaWVzIHRvIG9mZnNldCBwb2xsdXRpb24gb3IgcHJvbW90ZSB0YXJnZXRlZCBCTVAgYWRvcHRpb24gYXJlIGJlY29taW5nIHBvcHVsYXIgYW5kIHJlc3VsdGluZyBpbiBjb3N0LWVmZmVjdGl2ZSBhbmQgZWZmaWNpZW50IGFwcHJvYWNoZXMuIENvc3TigJNiZW5lZml0IGFuYWx5c2VzIG1heSBpbmNyZWFzZSB0aGUgdXNlIG9mIHRlY2huaWNhbCBzb2wtIHV0aW9ucyBvdmVyIGNvbXByZWhlbnNpdmUgYXBwcm9hY2hlcy4gQXQgdGhpcyBwb2ludCwgdGhlcmUgaXMgbm90IHJlYWxseSBhIHVuaXF1ZSBjYXNlIHN0dWR5IHRoYXQgY291bGQgYmUgYXBwbGllZCB0byBMYWtlIENoYW1wbGFpbiBhbmQgTGFrZSBNZW1waHJlbWFnb2cuIFNwZWNpZmljIGVsZW1lbnRzIG9mIHRoZSBhcHByb2FjaGVzIGlkZW50aWZpZWQgaW4gdGhpcyByZXZpZXcgYW5kIHVuaS0gZnlpbmcgdGhlbWVzLCBob3dldmVyLCBob2xkIHByb21pc2VzLiBUaGlzIGlzIHdoeSB3ZSBhZHZvY2F0ZSBmb3IgY29uZHVjdGluZyBhIHN5c3RlbWF0aWMgbGl0ZXJhdHVyZSByZXZpZXcgd2l0aCByZWdhcmQgdG8gdGhvc2UgdGhlbWVzIGluIG9yZGVyIHRvIGRldmVsb3AgcmVjb21tZW4tIGRhdGlvbnMgdGFpbG9yZWQgdG8gdGhlIExha2UgTWVtcGhyZW1hZ29nIGFuZCBNaXNzaXNxdW9pIEJheS1MYWtlIENoYW1wbGFpbiBhcmVhcy4gQXQgbGFzdCwgd2Ugd291bGQgbGlrZSB0byBhZGRyZXNzIHRoZSBlbGVwaGFudC1zaXplZCBxdWVzLSB0aW9uIGluIHRoZSByb29tLiBXaWxsIGNsaW1hdGUgY2hhbmdlIGluY3JlYXNlIEhBQnMgZnJlcXVlbmN5PyBJbnRlcmFjdGl2ZSBlZmZlY3RzIG9mIGV1dHJvcGhpY2F0aW9uIGFuZCBjbGltYXRlIGNoYW5nZSBvbiBoYXJtZnVsIGN5YW5vYmFjdGVyaWFsIGJsb29tcyBhcmUgY29tcGxleC4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjE1LCJiYm94Ijp7InRvcCI6ODUuNjA1NTE0NTI2MzY3MTksImxlZnQiOjQ5Ljc5NDg3MjI4MzkzNTU1LCJ3aWR0aCI6MzkuNzE4NjYyMjYxOTYyODksImhlaWdodCI6MS4zMjg2NzEzMzYxNzQwMTEyfX0seyJwYWdlTnVtYmVyIjoxNSwiYmJveCI6eyJ0b3AiOjg3LjM5NTM2Mjg1NDAwMzksImxlZnQiOjQ5Ljc5NDg3MjI4MzkzNTU1LCJ3aWR0aCI6MzkuNzAyNzA5MTk3OTk4MDUsImhlaWdodCI6MS4zMjg2NzEzMzYxNzQwMTEyfX0seyJwYWdlTnVtYmVyIjoxNSwiYmJveCI6eyJ0b3AiOjg5LjE4NTIxMTE4MTY0MDYyLCJsZWZ0Ijo0OS43OTQ4NzIyODM5MzU1NSwid2lkdGgiOjM5LjcwMjA5NTAzMTczODI4LCJoZWlnaHQiOjEuMzI4NjcxMzM2MTc0MDExMn19LHsicGFnZU51bWJlciI6MTUsImJib3giOnsidG9wIjo5MC45NzUwNTk1MDkyNzczNCwibGVmdCI6NDkuNzk0ODcyMjgzOTM1NTUsIndpZHRoIjozOS43MDQ2MjAzNjEzMjgxMjUsImhlaWdodCI6MS4zMjg2NzEzMzYxNzQwMTEyfX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjE0LjAyNTk5MDQ4NjE0NTAyLCJsZWZ0IjoxMC41MDIzMjMxNTA2MzQ3NjYsIndpZHRoIjozOS43MDU5NTU1MDUzNzEwOTQsImhlaWdodCI6MS4zMjg2Njk0Mjg4MjUzNzg0fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjE1LjgxNTg0MTY3NDgwNDY4OCwibGVmdCI6MTAuNTAyMzIzMTUwNjM0NzY2LCJ3aWR0aCI6MzkuNzAyMjQ3NjE5NjI4OTA2LCJoZWlnaHQiOjEuMzI4NjY5NDI4ODI1Mzc4NH19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjoxNy42MDU2OTE5MDk3OTAwNCwibGVmdCI6MTAuNTAyMTc2Mjg0NzkwMDM5LCJ3aWR0aCI6MzkuNzAwOTIzOTE5Njc3NzM0LCJoZWlnaHQiOjEuMzI4NjY5NDI4ODI1Mzc4NH19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjoxOS4zOTU1NDQwNTIxMjQwMjMsImxlZnQiOjEwLjUwMjE3NjI4NDc5MDAzOSwid2lkdGgiOjM5LjcwODkwMDQ1MTY2MDE1NiwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MjEuMTg1Mzk0Mjg3MTA5Mzc1LCJsZWZ0IjoxMC41MDIxNzYyODQ3OTAwMzksIndpZHRoIjozOS43MTAwODMwMDc4MTI1LCJoZWlnaHQiOjEuMzI4NjY5NDI4ODI1Mzc4NH19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjoyMi45NzUyNDQ1MjIwOTQ3MjcsImxlZnQiOjEwLjUwMjE3NjI4NDc5MDAzOSwid2lkdGgiOjM5LjcwNDAyOTA4MzI1MTk1LCJoZWlnaHQiOjEuMzI4NjY5NDI4ODI1Mzc4NH19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjoyNC43NjUwOTY2NjQ0Mjg3MSwibGVmdCI6MTAuNTAyMTc2Mjg0NzkwMDM5LCJ3aWR0aCI6MzkuNzAzODgwMzEwMDU4NTk0LCJoZWlnaHQiOjEuMzI4NjY5NDI4ODI1Mzc4NH19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjoyNi41NTQ5NDg4MDY3NjI2OTUsImxlZnQiOjEwLjUwMjAyNzUxMTU5NjY4LCJ3aWR0aCI6MzkuNzAzODgwMzEwMDU4NTk0LCJoZWlnaHQiOjEuMzI4NjY5NDI4ODI1Mzc4NH19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjoyOC4zNDQ3OTkwNDE3NDgwNDcsImxlZnQiOjEwLjUwMjAyNzUxMTU5NjY4LCJ3aWR0aCI6MzkuNzAwMDM4OTA5OTEyMTEsImhlaWdodCI6MS4zMjg2Njk0Mjg4MjUzNzg0fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjMwLjEzNDY0NzM2OTM4NDc2NiwibGVmdCI6MTAuNTAyMDI3NTExNTk2NjgsIndpZHRoIjozOS43MDQ2MTY1NDY2MzA4NiwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MzEuOTI0NDk3NjA0MzcwMTE3LCJsZWZ0IjoxMC41MDE4Nzc3ODQ3MjkwMDQsIndpZHRoIjozOS43MDU3ODc2NTg2OTE0MDYsImhlaWdodCI6MS4zMjg2Njk0Mjg4MjUzNzg0fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjMzLjcxNDM1MTY1NDA1MjczNCwibGVmdCI6MTAuNTAxNzI5MDExNTM1NjQ1LCJ3aWR0aCI6MzkuNzE5MjQ1OTEwNjQ0NTMsImhlaWdodCI6MS4zMjg2Njc0MDIyNjc0NTZ9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MzUuNTA0MjAzNzk2Mzg2NzIsImxlZnQiOjEwLjUwMTcyOTAxMTUzNTY0NSwid2lkdGgiOjM5LjcwMjcwOTE5Nzk5ODA1LCJoZWlnaHQiOjEuMzI4NjY3NDAyMjY3NDU2fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjM3LjI5NDA1MjEyNDAyMzQ0LCJsZWZ0IjoxMC41MDE3MjkwMTE1MzU2NDUsIndpZHRoIjozOS43MDU3OTkxMDI3ODMyLCJoZWlnaHQiOjEuMzI4NjY3NDAyMjY3NDU2fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjM5LjA4MzkwNDI2NjM1NzQyLCJsZWZ0IjoxMC41MDE3MjUxOTY4MzgzNzksIndpZHRoIjozOS43MDY1MzUzMzkzNTU0NywiaGVpZ2h0IjoxLjMyODY2NzQwMjI2NzQ1Nn19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjo0MC44NzM3NTY0MDg2OTE0MDYsImxlZnQiOjEwLjUwMTcyNzEwNDE4NzAxMiwid2lkdGgiOjM5LjcyMTE1MzI1OTI3NzM0NCwiaGVpZ2h0IjoxLjMyODY2NzQwMjI2NzQ1Nn19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjo0Mi42NjM2MDg1NTEwMjUzOSwibGVmdCI6MTAuNTAxNzI3MTA0MTg3MDEyLCJ3aWR0aCI6MzkuNzAzMTM2NDQ0MDkxOCwiaGVpZ2h0IjoxLjMyODY2NzQwMjI2NzQ1Nn19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjo0NC40NDYyMDg5NTM4NTc0MiwibGVmdCI6MTAuNTAxNzI3MTA0MTg3MDEyLCJ3aWR0aCI6MzkuNzAwNzc1MTQ2NDg0Mzc1LCJoZWlnaHQiOjEuMzI4NjY3NDAyMjY3NDU2fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjQ2LjIzNjA2MTA5NjE5MTQwNiwibGVmdCI6MTAuNTAxNzI3MTA0MTg3MDEyLCJ3aWR0aCI6MzkuNzA3NzI1NTI0OTAyMzQ0LCJoZWlnaHQiOjEuMzI4NjY3NDAyMjY3NDU2fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjQ4LjAyNTkxMzIzODUyNTM5LCJsZWZ0IjoxMC41MDE3MjcxMDQxODcwMTIsIndpZHRoIjoyNS45NDE2Njc1NTY3NjI2OTUsImhlaWdodCI6MS4zMjg2Njc0MDIyNjc0NTZ9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6NDkuODE1NzYxNTY2MTYyMTEsImxlZnQiOjEzLjEyNTAxNjIxMjQ2MzM3OSwid2lkdGgiOjM3LjA4MTE4MDU3MjUwOTc2NiwiaGVpZ2h0IjoxLjMyODY2NzQwMjI2NzQ1Nn19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjo1MS42MDU2MTM3MDg0OTYwOTQsImxlZnQiOjEwLjUwMTcyNzEwNDE4NzAxMiwid2lkdGgiOjM5LjcwOTIwNTYyNzQ0MTQwNiwiaGVpZ2h0IjoxLjMyODY2NzQwMjI2NzQ1Nn19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjo1My4zOTU0NjIwMzYxMzI4MSwibGVmdCI6MTAuNTAxNzI3MTA0MTg3MDEyLCJ3aWR0aCI6MzkuNzE5MjIzMDIyNDYwOTQsImhlaWdodCI6MS4zMjg2Njc0MDIyNjc0NTZ9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6NTUuMTg1MzEwMzYzNzY5NTMsImxlZnQiOjEwLjUwMTcyNzEwNDE4NzAxMiwid2lkdGgiOjM5LjcxMDY3ODEwMDU4NTk0LCJoZWlnaHQiOjEuMzI4NjY3NDAyMjY3NDU2fX1dfSwiY2h1bmtJZCI6ImRhM2VkNmVlLWViNGItNDQ4MC04MTRmLWE0ZGZlY2Y2MTIwNCIsImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjE2In19XQ%3D%3D.

11. Existing Regulatory Frameworks and Their Limitations

India's regulatory landscape for managing water quality and mitigating harmful algal blooms in lakes is anchored in several national policies and programs, yet it grapples with persistent implementation challenges. The National Water Policy of 2012 emphasizes integrated water resource management, prioritizing ecological sustainability and pollution control through measures like watershed development and groundwater regulation (Khorasani & Zhu, 2021)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhhbWVkIiwiZmFtaWx5IjoiS2hvcmFzYW5pIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlpoZW5kdW8iLCJmYW1pbHkiOiJaaHUifX1dLCJpZHMiOnsiaXNzbiI6WyIwMDIyLTE2OTQiLCIxODc5LTI3MDciXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMTYvai5qaHlkcm9sLjIwMjEuMTI2ODg2IiwibWFnIjoiMzE3NzA3MTI5OCIsIm9wZW5hbGV4IjoiVzMxNzcwNzEyOTgifSwidGl0bGUiOiJQaG9zcGhvcnVzIHJldGVudGlvbiBpbiBsYWtlczogQSBjcml0aWNhbCByZWFzc2Vzc21lbnQgb2YgaHlwb3RoZXNlcyBhbmQgc3RhdGljIG1vZGVscyIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2ouamh5ZHJvbC4yMDIxLjEyNjg4NiIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9lYXJ0aGFyeGl2Lm9yZy9yZXBvc2l0b3J5L29iamVjdC8yNDgxL2Rvd25sb2FkLzUwNzQvIiwiaXNzdWVkRGF0ZSI6IjIwMjEtMDktMDIiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA4IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJKb3VybmFsIG9mIEh5ZHJvbG9neSIsInZvbHVtZSI6NjAzLCJpbXBhY3RGYWN0b3IiOiI3LjY0In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTI2ODg2LCJsYXN0UGFnZSI6MTI2ODg2fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkVsc2V2aWVyIEJWIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo3NywiY2l0ZWRCeUNvdW50IjoxMn19LCJxdW90ZSI6Ikhvd2V2ZXIsIGh1bWFuIGFjdGl2aXRpZXMgaGF2ZSA0OSBhY2NlbGVyYXRlZCB0aGUgZXV0cm9waGljYXRpb24gcHJvY2VzcyBieSBpbmNyZWFzaW5nIHRoZSBudXRyaWVudHMgZGVsaXZlcnkgdG8gdGhlIGFxdWF0aWMgc3lzdGVtcyA1MCAoTWVrb25uZW4gYW5kIEhvZWtzdHJhLCAyMDE4KS4gVGh1cywgYW50aHJvcG9nZW5pYyBldXRyb3BoaWNhdGlvbiBpcyBvbmUgb2YgdGhlIG1vc3QgaW1wb3J0YW50IDUxIGVsZW1lbnRzIG9mIGZyZXNoIGFuZCBtYXJpbmUgd2F0ZXIgcXVhbGl0eSBkZXRlcmlvcmF0aW9uIChIdSBldCBhbC4sIDIwMTk7IFNtaXRoIGFuZCBTY2hpbmRsZXIsIDUyIDIwMDkpLiBPbmUgZGlyZWN0IGNvbnNlcXVlbmNlIG9mIGFudGhyb3BvZ2VuaWMgZXV0cm9waGljYXRpb24gY29tZXMgaW4gdGhlIGZvcm0gb2YgbWFzc2l2ZSBhbGdhbCA1MyBibG9vbXMgKEdyYW7DqWxpIGV0IGFsLiwgMjAwODsgSGVpc2xlciBldCBhbC4sIDIwMDgpLCB3aGljaCBhcmUgcHJlZGljdGVkIHRvIGJlIGludGVuc2lmaWVkIHVuZGVyIDU0IHdhcm1lciB3YXRlciB0ZW1wZXJhdHVyZXMgYXMgY2xpbWF0ZSBjaGFuZ2VzIChHb2JsZXIsIDIwMjA7IE11a3VuZGFuIGV0IGFsLiwgMjAyMCkuIDU1IEV1dHJvcGhpY2F0aW9uIGlzIGEg4oCcd2lja2Vk4oCdIHByb2JsZW0sIHdoaWNoIGlzIHRoZSBjb25zZXF1ZW5jZSBvZiB2YXJpb3VzIHByb2Nlc3NlcyB0aGF0IG9wZXJhdGUgNTYgY3VtdWxhdGl2ZWx5LiBDb25zaWRlcmluZyB0aGUgdW5pcXVlbmVzcyBvZiBlYWNoIGxha2UgYW5kIGl0cyBzdXJyb3VuZGluZyBhcmVhLCB0aGVyZSBpcyBubyBicm9hZGx5IDU3IGFwcGxpY2FibGUgc2V0IG9mIGJlc3QgbWFuYWdlbWVudCBwcmFjdGljZXMgdGhhdCBjYW4gYmUgYXBwbGllZCBpbiB3YXRlcnNoZWRzIHRvIG1pdGlnYXRlIDU4IHBob3NwaG9ydXMgbG9hZGluZyBhbmQgaXRzIGltcGFjdCBvbiBhbGwgbGFrZXMgKFRob3JudG9uIGV0IGFsLiwgMjAxMykuIEhlbmNlLCBldXRyb3BoaWNhdGlvbiA1OSBtYW5hZ2VtZW50IGFuZCBsYWtlIHJlc3RvcmF0aW9uIG5lZWQgaW50ZWdyYXRlZCBwbGFucyB0aGF0IGFyZSBub3Qgb25seSBzY2llbnRpZmljYWxseSB2YWxpZCBidXQgNjAgYWxzbyBzb2Npby1lY29ub21pY2FsbHkgc2F0aXNmeWluZyAoR2lic29uIGV0IGFsLiwgMjAwMCkuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MzYuMTY4NDM0MTQzMDY2NDA2LCJsZWZ0IjoxMS43Njg2MjgxMjA0MjIzNjMsIndpZHRoIjo3Ni45NjMzOTQxNjUwMzkwNiwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjM2LjcxODQzMzM4MDEyNjk1LCJsZWZ0Ijo2Ljk4MDM5MjQ1NjA1NDY4NzUsIndpZHRoIjoyLjI1MDk1NzQ4OTAxMzY3MiwiaGVpZ2h0IjoxLjM5Mzk0MDQ0ODc2MDk4NjN9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjozOS42NTMyODU5ODAyMjQ2MSwibGVmdCI6Ni45ODAzOTI0NTYwNTQ2ODc1LCJ3aWR0aCI6ODEuNzYxNDI4ODMzMDA3ODEsImhlaWdodCI6Mi4wMjEyMTE2MjQxNDU1MDh9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo0My4xMzgxMzQwMDI2ODU1NSwibGVmdCI6Ni45ODAzOTI0NTYwNTQ2ODc1LCJ3aWR0aCI6ODEuNzU1NTYxODI4NjEzMjgsImhlaWdodCI6Mi4wMjEyMTE2MjQxNDU1MDh9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo0Ni42MjI5ODIwMjUxNDY0ODQsImxlZnQiOjYuOTgwMzkyNDU2MDU0Njg3NSwid2lkdGgiOjgxLjc0MjgwNTQ4MDk1NzAzLCJoZWlnaHQiOjIuMDIxMjExNjI0MTQ1NTA4fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6NTAuMTA3ODMwMDQ3NjA3NDIsImxlZnQiOjYuOTgwMzkyNDU2MDU0Njg3NSwid2lkdGgiOjgxLjc1MjYxNjg4MjMyNDIyLCJoZWlnaHQiOjIuMDIxMjExNjI0MTQ1NTA4fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6NTMuNTk1MjAzMzk5NjU4MiwibGVmdCI6Ni45ODAzOTI0NTYwNTQ2ODc1LCJ3aWR0aCI6ODEuNzQ4NzAzMDAyOTI5NjksImhlaWdodCI6Mi4wMjEyMTE2MjQxNDU1MDh9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo1Ny4wODAwNDc2MDc0MjE4NzUsImxlZnQiOjYuOTgwMzkyNDU2MDU0Njg3NSwid2lkdGgiOjczLjMxMDQ1NTMyMjI2NTYyLCJoZWlnaHQiOjIuMDIxMjExNjI0MTQ1NTA4fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6NjEuNTY0OTAzMjU5Mjc3MzQ0LCJsZWZ0Ijo2Ljk4MDM5MjQ1NjA1NDY4NzUsIndpZHRoIjo4MS43NTc1MTQ5NTM2MTMyOCwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjY1LjA0OTc0MzY1MjM0Mzc1LCJsZWZ0Ijo2Ljk4MDM5MjQ1NjA1NDY4NzUsIndpZHRoIjo4MS43NTk0NzU3MDgwMDc4MSwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjY4LjUzNDU5MTY3NDgwNDY5LCJsZWZ0Ijo2Ljk4MDM5MjQ1NjA1NDY4NzUsIndpZHRoIjo4MS43MDM5MzM3MTU4MjAzMSwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjcyLjAxOTQ1NDk1NjA1NDY5LCJsZWZ0Ijo2Ljk4MDM5MjQ1NjA1NDY4NzUsIndpZHRoIjo4MS43NTA2NTYxMjc5Mjk2OSwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjc1LjUwODA4NzE1ODIwMzEyLCJsZWZ0Ijo2Ljk4MDM5MjQ1NjA1NDY4NzUsIndpZHRoIjo4MS43NTc1MTQ5NTM2MTMyOCwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjc4Ljk5MjkzNTE4MDY2NDA2LCJsZWZ0IjoxMS43Njg2MjgxMjA0MjIzNjMsIndpZHRoIjo3Ni45NzMxOTc5MzcwMTE3MiwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEyNjg4OCJ9fV0%3D. Complementing this, the National Lake Conservation Plan, launched under the Ministry of Environment, Forest and Climate Change, targets restoration of 63 priority lakes via sewage diversion, desilting, and bioremediation, with a budget exceeding INR 1,000 crore by 2020. The Wetlands Rules, 2017, mandate state-level wetland authorities to regulate activities in notified lakes, prohibiting encroachments and solid waste dumping. Furthermore, the National Mission for Clean Ganga extends principles to inland waters, promoting nutrient reduction via advanced wastewater treatment. These frameworks recognize eutrophication as a "wicked" problem driven by cumulative nutrient loads from sewage and agriculture, advocating best management practices like riparian buffers (Khorasani & Zhu, 2021)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhhbWVkIiwiZmFtaWx5IjoiS2hvcmFzYW5pIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlpoZW5kdW8iLCJmYW1pbHkiOiJaaHUifX1dLCJpZHMiOnsiaXNzbiI6WyIwMDIyLTE2OTQiLCIxODc5LTI3MDciXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMTYvai5qaHlkcm9sLjIwMjEuMTI2ODg2IiwibWFnIjoiMzE3NzA3MTI5OCIsIm9wZW5hbGV4IjoiVzMxNzcwNzEyOTgifSwidGl0bGUiOiJQaG9zcGhvcnVzIHJldGVudGlvbiBpbiBsYWtlczogQSBjcml0aWNhbCByZWFzc2Vzc21lbnQgb2YgaHlwb3RoZXNlcyBhbmQgc3RhdGljIG1vZGVscyIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2ouamh5ZHJvbC4yMDIxLjEyNjg4NiIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9lYXJ0aGFyeGl2Lm9yZy9yZXBvc2l0b3J5L29iamVjdC8yNDgxL2Rvd25sb2FkLzUwNzQvIiwiaXNzdWVkRGF0ZSI6IjIwMjEtMDktMDIiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA4IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJKb3VybmFsIG9mIEh5ZHJvbG9neSIsInZvbHVtZSI6NjAzLCJpbXBhY3RGYWN0b3IiOiI3LjY0In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTI2ODg2LCJsYXN0UGFnZSI6MTI2ODg2fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkVsc2V2aWVyIEJWIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo3NywiY2l0ZWRCeUNvdW50IjoxMn19LCJxdW90ZSI6Ikhvd2V2ZXIsIGh1bWFuIGFjdGl2aXRpZXMgaGF2ZSA0OSBhY2NlbGVyYXRlZCB0aGUgZXV0cm9waGljYXRpb24gcHJvY2VzcyBieSBpbmNyZWFzaW5nIHRoZSBudXRyaWVudHMgZGVsaXZlcnkgdG8gdGhlIGFxdWF0aWMgc3lzdGVtcyA1MCAoTWVrb25uZW4gYW5kIEhvZWtzdHJhLCAyMDE4KS4gVGh1cywgYW50aHJvcG9nZW5pYyBldXRyb3BoaWNhdGlvbiBpcyBvbmUgb2YgdGhlIG1vc3QgaW1wb3J0YW50IDUxIGVsZW1lbnRzIG9mIGZyZXNoIGFuZCBtYXJpbmUgd2F0ZXIgcXVhbGl0eSBkZXRlcmlvcmF0aW9uIChIdSBldCBhbC4sIDIwMTk7IFNtaXRoIGFuZCBTY2hpbmRsZXIsIDUyIDIwMDkpLiBPbmUgZGlyZWN0IGNvbnNlcXVlbmNlIG9mIGFudGhyb3BvZ2VuaWMgZXV0cm9waGljYXRpb24gY29tZXMgaW4gdGhlIGZvcm0gb2YgbWFzc2l2ZSBhbGdhbCA1MyBibG9vbXMgKEdyYW7DqWxpIGV0IGFsLiwgMjAwODsgSGVpc2xlciBldCBhbC4sIDIwMDgpLCB3aGljaCBhcmUgcHJlZGljdGVkIHRvIGJlIGludGVuc2lmaWVkIHVuZGVyIDU0IHdhcm1lciB3YXRlciB0ZW1wZXJhdHVyZXMgYXMgY2xpbWF0ZSBjaGFuZ2VzIChHb2JsZXIsIDIwMjA7IE11a3VuZGFuIGV0IGFsLiwgMjAyMCkuIDU1IEV1dHJvcGhpY2F0aW9uIGlzIGEg4oCcd2lja2Vk4oCdIHByb2JsZW0sIHdoaWNoIGlzIHRoZSBjb25zZXF1ZW5jZSBvZiB2YXJpb3VzIHByb2Nlc3NlcyB0aGF0IG9wZXJhdGUgNTYgY3VtdWxhdGl2ZWx5LiBDb25zaWRlcmluZyB0aGUgdW5pcXVlbmVzcyBvZiBlYWNoIGxha2UgYW5kIGl0cyBzdXJyb3VuZGluZyBhcmVhLCB0aGVyZSBpcyBubyBicm9hZGx5IDU3IGFwcGxpY2FibGUgc2V0IG9mIGJlc3QgbWFuYWdlbWVudCBwcmFjdGljZXMgdGhhdCBjYW4gYmUgYXBwbGllZCBpbiB3YXRlcnNoZWRzIHRvIG1pdGlnYXRlIDU4IHBob3NwaG9ydXMgbG9hZGluZyBhbmQgaXRzIGltcGFjdCBvbiBhbGwgbGFrZXMgKFRob3JudG9uIGV0IGFsLiwgMjAxMykuIEhlbmNlLCBldXRyb3BoaWNhdGlvbiA1OSBtYW5hZ2VtZW50IGFuZCBsYWtlIHJlc3RvcmF0aW9uIG5lZWQgaW50ZWdyYXRlZCBwbGFucyB0aGF0IGFyZSBub3Qgb25seSBzY2llbnRpZmljYWxseSB2YWxpZCBidXQgNjAgYWxzbyBzb2Npby1lY29ub21pY2FsbHkgc2F0aXNmeWluZyAoR2lic29uIGV0IGFsLiwgMjAwMCkuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MzYuMTY4NDM0MTQzMDY2NDA2LCJsZWZ0IjoxMS43Njg2MjgxMjA0MjIzNjMsIndpZHRoIjo3Ni45NjMzOTQxNjUwMzkwNiwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjM2LjcxODQzMzM4MDEyNjk1LCJsZWZ0Ijo2Ljk4MDM5MjQ1NjA1NDY4NzUsIndpZHRoIjoyLjI1MDk1NzQ4OTAxMzY3MiwiaGVpZ2h0IjoxLjM5Mzk0MDQ0ODc2MDk4NjN9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjozOS42NTMyODU5ODAyMjQ2MSwibGVmdCI6Ni45ODAzOTI0NTYwNTQ2ODc1LCJ3aWR0aCI6ODEuNzYxNDI4ODMzMDA3ODEsImhlaWdodCI6Mi4wMjEyMTE2MjQxNDU1MDh9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo0My4xMzgxMzQwMDI2ODU1NSwibGVmdCI6Ni45ODAzOTI0NTYwNTQ2ODc1LCJ3aWR0aCI6ODEuNzU1NTYxODI4NjEzMjgsImhlaWdodCI6Mi4wMjEyMTE2MjQxNDU1MDh9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo0Ni42MjI5ODIwMjUxNDY0ODQsImxlZnQiOjYuOTgwMzkyNDU2MDU0Njg3NSwid2lkdGgiOjgxLjc0MjgwNTQ4MDk1NzAzLCJoZWlnaHQiOjIuMDIxMjExNjI0MTQ1NTA4fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6NTAuMTA3ODMwMDQ3NjA3NDIsImxlZnQiOjYuOTgwMzkyNDU2MDU0Njg3NSwid2lkdGgiOjgxLjc1MjYxNjg4MjMyNDIyLCJoZWlnaHQiOjIuMDIxMjExNjI0MTQ1NTA4fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6NTMuNTk1MjAzMzk5NjU4MiwibGVmdCI6Ni45ODAzOTI0NTYwNTQ2ODc1LCJ3aWR0aCI6ODEuNzQ4NzAzMDAyOTI5NjksImhlaWdodCI6Mi4wMjEyMTE2MjQxNDU1MDh9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo1Ny4wODAwNDc2MDc0MjE4NzUsImxlZnQiOjYuOTgwMzkyNDU2MDU0Njg3NSwid2lkdGgiOjczLjMxMDQ1NTMyMjI2NTYyLCJoZWlnaHQiOjIuMDIxMjExNjI0MTQ1NTA4fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6NjEuNTY0OTAzMjU5Mjc3MzQ0LCJsZWZ0Ijo2Ljk4MDM5MjQ1NjA1NDY4NzUsIndpZHRoIjo4MS43NTc1MTQ5NTM2MTMyOCwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjY1LjA0OTc0MzY1MjM0Mzc1LCJsZWZ0Ijo2Ljk4MDM5MjQ1NjA1NDY4NzUsIndpZHRoIjo4MS43NTk0NzU3MDgwMDc4MSwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjY4LjUzNDU5MTY3NDgwNDY5LCJsZWZ0Ijo2Ljk4MDM5MjQ1NjA1NDY4NzUsIndpZHRoIjo4MS43MDM5MzM3MTU4MjAzMSwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjcyLjAxOTQ1NDk1NjA1NDY5LCJsZWZ0Ijo2Ljk4MDM5MjQ1NjA1NDY4NzUsIndpZHRoIjo4MS43NTA2NTYxMjc5Mjk2OSwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjc1LjUwODA4NzE1ODIwMzEyLCJsZWZ0Ijo2Ljk4MDM5MjQ1NjA1NDY4NzUsIndpZHRoIjo4MS43NTc1MTQ5NTM2MTMyOCwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjc4Ljk5MjkzNTE4MDY2NDA2LCJsZWZ0IjoxMS43Njg2MjgxMjA0MjIzNjMsIndpZHRoIjo3Ni45NzMxOTc5MzcwMTE3MiwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEyNjg4OCJ9fV0%3D. However, their efficacy remains curtailed by fragmented enforcement.

At the state level, policies adapt national directives to regional contexts but often mirror top-down deficiencies. Karnataka's Lake Conservation and Development Authority Act addresses Bellandur Lake's frothing crises by mandating zero-liquid discharge from industries and stormwater drains, yet recurrent blooms underscore gaps (Jansen et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkpvYWNoaW0iLCJmYW1pbHkiOiJKYW5zZW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiR2F2aW4gTC4iLCJmYW1pbHkiOiJTaW1wc29uIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikdlc2EgQS4iLCJmYW1pbHkiOiJXZXloZW5tZXllciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJMYXVyYSBILiIsImZhbWlseSI6IkjDpHJrw7ZuZW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW5kcmV3IE0uIiwiZmFtaWx5IjoiUGF0ZXJzb24ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUGF1bCBBLiBkZWwiLCJmYW1pbHkiOiJHaW9yZ2lvIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ill2ZXMgVC4iLCJmYW1pbHkiOiJQcmFpcmllIn19XSwiaWRzIjp7Imlzc24iOlsiMTc1OC02NzhYIiwiMTc1OC02Nzk4Il0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDM4L3M0MTU1OC0wMjQtMDIwNTgtMyIsIm9wZW5hbGV4IjoiVzQ0MDAyMDgyMDAiLCJleGEiOiJodHRwczovL3d3dy5uYXR1cmUuY29tL2FydGljbGVzL3M0MTU1OC0wMjQtMDIwNTgtMz9lcnJvcj1jb29raWVzX25vdF9zdXBwb3J0ZWQmY29kZT1hNzRhMjVmZi1hNThkLTRkZjgtOTM2Zi0yNjJlMmUzZTkxOTgifSwidGl0bGUiOiJDbGltYXRlLWRyaXZlbiBkZW94eWdlbmF0aW9uIG9mIG5vcnRoZXJuIGxha2VzIiwiYWJzdHJhY3QiOiJBYnN0cmFjdCBPeHlnZW4gZGVwbGV0aW9uIGNvbnN0aXR1dGVzIGEgbWFqb3IgdGhyZWF0IHRvIGxha2UgZWNvc3lzdGVtcyBhbmQgdGhlIHNlcnZpY2VzIHRoZXkgcHJvdmlkZS4gTW9zdCBvZiB0aGUgd29ybGTigJlzIGxha2VzIGFyZSBsb2NhdGVkICZndDs0NcKwIE4sIHdoZXJlIGFjY2VsZXJhdGVkIGNsaW1hdGUgd2FybWluZyBhbmQgZWxldmF0ZWQgY2FyYm9uIGxvYWRzIG1pZ2h0IHNldmVyZWx5IGluY3JlYXNlIHRoZSByaXNrIG9mIGh5cG94aWEsIGJ1dCB0aGlzIGhhcyBub3QgYmVlbiBzeXN0ZW1hdGljYWxseSBleGFtaW5lZC4gSGVyZSBhbmFseXNpcyBvZiAyLjYgbWlsbGlvbiB3YXRlciBxdWFsaXR5IG9ic2VydmF0aW9ucyBmcm9tIDgsMjg4IGxha2VzIHNob3dzIHRoYXQgYmV0d2VlbiAxOTYwIGFuZCAyMDIyLCBtb3N0IG5vcnRoZXJuIGxha2VzIGV4cGVyaWVuY2VkIHJhcGlkIGRlb3h5Z2VuYXRpb24gc3Ryb25nbHkgbGlua2VkIHRvIGNsaW1hdGUtZHJpdmVuIHByb2xvbmdhdGlvbiBvZiBzdW1tZXIgc3RyYXRpZmljYXRpb24uIE94eWdlbiBsZXZlbHMgZGV0ZXJpb3JhdGVkIG1vc3QgaW4gc21hbGwgbGFrZXMgKCZsdDsxMCBoYSkgb3dpbmcgdG8gdGhlaXIgZ3JlYXRlciB2b2x1bWV0cmljIG94eWdlbiBkZW1hbmQgYW5kIHN1cmZhY2Ugd2FybWluZyByYXRlcywgd2hpbGUgdGhlIGxhcmdlc3QgbGFrZXMgZ2FpbmVkIG94eWdlbiB1bmRlciBtaW5pbWFsIHN0cmF0aWZpY2F0aW9uIGNoYW5nZXMgYW5kIGltcHJvdmVkIGFlcmF0aW9uIGF0IHNwcmluZyBvdmVydHVybnMuIFNlYXNvbmFsIG94eWdlbiBjb25zdW1wdGlvbiByYXRlcyBkZWNsaW5lZCwgZGVzcGl0ZSB3aWRlc3ByZWFkIGJyb3duaW5nLiBQcm9saWZlcmF0aW5nIGFub3hpYSBlbmhhbmNlZCBzZWFzb25hbCBpbnRlcm5hbCBsb2FkaW5nIG9mIEMsIFAgYW5kIE4gYnV0IGRlcGxldGVkIFAgbG9uZy10ZXJtLCBpbmRpY2F0aW5nIHRoYXQgZGVveHlnZW5hdGlvbiBjYW4gZXhoYXVzdCByZWRveC1zZW5zaXRpdmUgZnJhY3Rpb25zIG9mIHNlZGltZW50IG51dHJpZW50IHJlc2Vydm9pcnMuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjEwMzgvczQxNTU4LTAyNC0wMjA1OC0zIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAzOC9zNDE1NTgtMDI0LTAyMDU4LTMiLCJpc3N1ZWREYXRlIjoiMjAyNC0wNy0wMSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTEiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6Ik5hdHVyZSBDbGltYXRlIENoYW5nZSIsImlzc3VlIjo4LCJ2b2x1bWUiOjE0LCJpbXBhY3RGYWN0b3IiOiIxNS4wMiJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjgzMiwibGFzdFBhZ2UiOjgzOH19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJOYXR1cmUgUG9ydGZvbGlvIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo4MCwiY2l0ZWRCeUNvdW50IjozN319LCJxdW90ZSI6IkFlcm9iaWMgcmVzcGlyYXRpb24gY2FuIGRlcGxldGUgb3h5Z2VuIGNvbmNlbnRyYXRpb25zIGF0IGRlcHRoIHdoZW4gc2Vhc29uYWwgc3RyYXRpZmljYXRpb24gcHJldmVudHMgYWVyYXRpb24gb2YgdGhlIHdhdGVyIGNvbHVtbls1XShodHRwczovL3d3dy5uYXR1cmUuY29tL2FydGljbGVzL3M0MTU1OC0wMjQtMDIwNTgtMyNyZWYtQ1I1KS4gVGhpcyBkZXRyaW1lbnRhbCBjb25kaXRpb24sIHRlcm1lZCBoeXBveGlhIChET%2BKAiTzigIky4oCJbWfigIls4oiSMSksIGRpbWluaXNoZXMgdGhlIGNvbGQtd2F0ZXIgaGFiaXRhdCBvZiBhcXVhdGljIGxpZmUsIGluY2x1ZGluZyBjb21tZXJjaWFsbHkgaW1wb3J0YW50IGZpc2hbNl0oaHR0cHM6Ly93d3cubmF0dXJlLmNvbS9hcnRpY2xlcy9zNDE1NTgtMDI0LTAyMDU4LTMjcmVmLUNSNikgYW5kIGNhbiBjYXVzZSBtYXNzIG1vcnRhbGl0eSBpZiBwZXJzaXN0ZW50WzddKGh0dHBzOi8vd3d3Lm5hdHVyZS5jb20vYXJ0aWNsZXMvczQxNTU4LTAyNC0wMjA1OC0zI3JlZi1DUjcpLiBBbm94aWEgKERP4oCJPOKAiTAuNeKAiW1n4oCJbOKIkjEpIHByb21vdGVzIHRoZSByZWR1Y3RpdmUgZGlzc29sdXRpb24gb2YgbWluZXJhbC1ib3VuZCBjYXJib24gYW5kIHBob3NwaG9ydXNbOF0oaHR0cHM6Ly93d3cubmF0dXJlLmNvbS9hcnRpY2xlcy9zNDE1NTgtMDI0LTAyMDU4LTMjcmVmLUNSOCksIFs5XShodHRwczovL3d3dy5uYXR1cmUuY29tL2FydGljbGVzL3M0MTU1OC0wMjQtMDIwNTgtMyNyZWYtQ1I5KSB3aGljaCBmdWVscyBoYXJtZnVsIGFsZ2FsIGJsb29tc1sxMF0oaHR0cHM6Ly93d3cubmF0dXJlLmNvbS9hcnRpY2xlcy9zNDE1NTgtMDI0LTAyMDU4LTMjcmVmLUNSMTApIGFuZCBsaW1pdHMgbWljcm9iaWFsIG94aWRhdGlvbiBvZiBtZXRoYW5lLCBhIHBvdGVudCBncmVlbmhvdXNlIGdhc1sxMV0oaHR0cHM6Ly93d3cubmF0dXJlLmNvbS9hcnRpY2xlcy9zNDE1NTgtMDI0LTAyMDU4LTMjcmVmLUNSMTEpLiJ9XQ%3D%3D. In Jammu & Kashmir, the Srinagar Master Plan integrates Dal Lake protection through dredging and sewage treatment plants, but tourism pressures overwhelm capacities, with total phosphorus exceeding 100 μg/L(Paerl et al., 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhhbnMgVy4iLCJmYW1pbHkiOiJQYWVybCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLLkUuIiwiZmFtaWx5IjoiSGF2ZW5zIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik4uUy4iLCJmYW1pbHkiOiJIYWxsIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlQuRy4iLCJmYW1pbHkiOiJPdHRlbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNLiIsImZhbWlseSI6IlpodSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJIYWlodWkiLCJmYW1pbHkiOiJYdSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJHLiIsImZhbWlseSI6IlpodSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJCLiIsImZhbWlseSI6IlFpbiJ9fV0sImlkcyI6eyJpc3NuIjpbXSwiaXNibiI6W10sImRvaSI6IjEwLjE3NjE1LzRua2Qtems5MyIsIm9wZW5hbGV4IjoiVzQzMDcwNjI0MjkifSwidGl0bGUiOiJNaXRpZ2F0aW5nIGEgZ2xvYmFsIGV4cGFuc2lvbiBvZiB0b3hpYyBjeWFub2JhY3RlcmlhbCBibG9vbXM6IENvbmZvdW5kaW5nIGVmZmVjdHMgYW5kIGNoYWxsZW5nZXMgcG9zZWQgYnkgY2xpbWF0ZSBjaGFuZ2UiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTc2MTUvNG5rZC16azkzIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2Nkci5saWIudW5jLmVkdS9kb3dubG9hZHMvMDU3NDIyODM0IiwiaXNzdWVkRGF0ZSI6IjIwMjAtMDEtMDEiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTAxIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJDYXJvbGluYSBEaWdpdGFsIFJlcG9zaXRvcnkgKFVuaXZlcnNpdHkgb2YgTm9ydGggQ2Fyb2xpbmEgYXQgQ2hhcGVsIEhpbGwpIn0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlVuaXZlcnNpdHkgb2YgTm9ydGggQ2Fyb2xpbmEgYXQgQ2hhcGVsIEhpbGwifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjAsImNpdGVkQnlDb3VudCI6MH19LCJxdW90ZSI6IlRoYXQgcmVsYXRpb25zaGlwIGhhcyBzZXJ2ZWQgYXMgdGhlIGZvdW5kYXRpb24gZm9yIHNldHRpbmcgIG51dHJpZW50IGxvYWRpbmcgdGFyZ2V0cyBpbiBsYWtlcyBhcm91bmQgdGhlIHdvcmxkLCBhbmQgY2FyZWZ1bGx5ICBkZXNpZ25lZCBleHBlcmltZW50cyBtYXkgZWx1Y2lkYXRlIHRvIHdoYXQgZXh0ZW50IHRlbXBlcmF0dXJlICBpbmNyZWFzZSBhZmZlY3RzIHRoZSB0aHJlc2hvbGQgYXQgd2hpY2ggdGhlIGZyZXF1ZW5jeSBvZiBvY2N1ciAgcmVuY2Ugb2YgQ3lhbm9IQUJzIHF1aWNrbHkgcmlzZXMgd2l0aCBpbmNyZWFzaW5nIE4gYW5kIFAuIENvbmNsdXNpb25zIFJlY2VudCBzdHVkaWVzIGluZGljYXRlIHRoYXQgdGhlIHN5bmVyZ2lzdGljIGNvbWJpbmF0aW9uIG9mIGFudGhyb3BvZ2VuaWMgbnV0cmllbnQgbG9hZGluZywgcmlzaW5nIHRlbXBlcmF0dXJlcywgZW5oYW5jZWQgIHZlcnRpY2FsIHN0cmF0aWZpY2F0aW9uLCBpbmNyZWFzZWQgcmVzaWRlbmNlIHRpbWUgYW5kIG1vcmUgIGV4dHJlbWUgY2xpbWF0aWMgY29uZGl0aW9ucyBvdmVyYWxsIHdpbGwgZmF2b3VyIGN5YW5vYmFjdGVyaWFsICBkb21pbmFuY2UgaW4gYSB3aWRlIHJhbmdlIG9mIGFxdWF0aWMgZWNvc3lzdGVtcy4gVGhlIGV4cGFuICBzaW9uIG9mIEN5YW5vSEFCcyBoYXMgc2VyaW91cyBjb25zZXF1ZW5jZXMgZm9yIGh1bWFuIHdhdGVyICBzdXBwbGllcywgZmlzaGVyaWVzIGFuZCByZWNyZWF0aW9uYWwgcmVzb3VyY2VzLiBFY29zeXN0ZW0gc2NhbGUgIHBoeXNpY2FsIGNoZW1pY2FsIGJpb3RpYyBtaXRpZ2F0aW9uIHN0cmF0ZWdpZXMgd2lsbCBoYXZlIHRvICBpbmNvcnBvcmF0ZSBudXRyaWVudCAobW9zdCBvZnRlbiBib3RoIE4gYW5kIFApIGlucHV0IHJlZHVjdGlvbnMuICBUaGUgbWFnbml0dWRlcyBvZiB0aGVzZSByZWR1Y3Rpb25zIHdpbGwgbmVlZCB0byBiZSBzeXN0ZW0gIHNwZWNpZmljIGFuZCB3aWxsIGxpa2VseSBuZWVkIHRvIGJlIGFkanVzdGVkIHdpdGggY2hhbmdpbmcgY2xpICBtYXRpYyBjb25kaXRpb25zLiBTcGVjaWZpY2FsbHksIHdhdGVyIHF1YWxpdHkgbWFuYWdlcnMgd2lsbCBoYXZlICB0byBhY2NvbW1vZGF0ZSB3YXJtaW5nIGFuZCBncmVhdGVyIGh5ZHJvbG9naWNhbCB2YXJpYWJpbGl0eSAgYW5kIGV4dHJlbWVuZXNzIHJlc3VsdGluZyBmcm9tIGNsaW1hdGUgY2hhbmdlIGluIGZvcm11bGF0aW5nICB0aGVpciBzdHJhdGVnaWVzLiBBIGtleSBsb25nIHRlcm0gY29udHJvbCB3ZSBjYW4gZXhlcnQgdG8gcmVkdWNlICB0aGUgcmF0ZSBhbmQgZXh0ZW50IG9mIGdsb2JhbCB3YXJtaW5nIGlzIGN1cmJpbmcgZ3JlZW5ob3VzZSBnYXMgIGVtaXNzaW9ucy4gV2l0aG91dCB0aGlzIGVzc2VudGlhbCBzdGVwLCBmdXR1cmUgd2FybWluZyB0cmVuZHMsICBoeWRyb2xvZ2ljYWwgZXh0cmVtZXMgKHJlY29yZCByYWluZmFsbHMgYW5kIGRyb3VnaHRzKSBhbmQgdGhlaXIgIGVmZmVjdHMgb24gYXF1YXRpYyBlY29zeXN0ZW1zIHdpbGwgY29udGludWUgdG8gcHJvbW90ZSBvcHBvciAgdHVuaXN0aWMgQ3lhbm9IQUJzLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjoxNC4xNDkyMzM4MTgwNTQyLCJsZWZ0IjoxMy4xMjg1OTUzNTIxNzI4NTIsIndpZHRoIjozNy4wODIxNzIzOTM3OTg4MywiaGVpZ2h0IjoxLjUxNjYwMjI3Nzc1NTczNzN9fSx7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6MTUuNDUxNTA4NTIyMDMzNjkxLCJsZWZ0IjoxMy4xMjg1NDQ4MDc0MzQwODIsIndpZHRoIjozNy4wODQ4MzEyMzc3OTI5NywiaGVpZ2h0IjoxLjUxNjYwMzIzMTQzMDA1Mzd9fSx7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6MTYuNzUzNzg0MTc5Njg3NSwibGVmdCI6MTMuMTI4NTQyOTAwMDg1NDUsIndpZHRoIjozNy4wODMyMDk5OTE0NTUwOCwiaGVpZ2h0IjoxLjUxNjYwNDE4NTEwNDM3MDF9fSx7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6MTguMDU2MDUzMTYxNjIxMDk0LCJsZWZ0IjoxMy4xMjg1NDQ4MDc0MzQwODIsIndpZHRoIjozNi42MTUwNzQxNTc3MTQ4NDQsImhlaWdodCI6MS41MTY2MDQxODUxMDQzNzAxfX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjE5LjQ2ODM3ODA2NzAxNjYsImxlZnQiOjEzLjEyODU0NDgwNzQzNDA4Miwid2lkdGgiOjM0LjI5MDUzMTE1ODQ0NzI2NiwiaGVpZ2h0IjoxLjI0NTAxMTIxMDQ0MTU4OTR9fSx7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6MjIuMjI2MTQyODgzMzAwNzgsImxlZnQiOjEzLjEyODU1MDUyOTQ3OTk4LCJ3aWR0aCI6Ny42NDk4NzM3MzM1MjA1MDgsImhlaWdodCI6MS4xMzU0ODMxNDU3MTM4MDYyfX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjIzLjczNDg0OTkyOTgwOTU3LCJsZWZ0IjoxMy4xMjg1MzUyNzA2OTA5MTgsIndpZHRoIjozNy4wODkyODY4MDQxOTkyMiwiaGVpZ2h0IjoyNi4wODg2NDAyMTMwMTI2OTV9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIxMSJ9fV0%3D. Odisha's Chilika Development Authority exemplifies hydrological interventions post-2000 inlet breach, reviving fisheries temporarily, though aquaculture effluents sustain cyanobacterial dominance (Jilbert et al., 2020; Nair & Nayak, 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hdnlhIFZpa3JhbWFuIiwiZmFtaWx5IjoiTmFpciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQcmF0ZWVwIEt1bWFyIiwiZmFtaWx5IjoiTmF5YWsifX1dLCJpZHMiOnsiaXNzbiI6WyIyMjk2LTc3NDUiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzODkvZm1hcnMuMjAyMy4xMDg3Mjk2Iiwib3BlbmFsZXgiOiJXNDM4MjcyMzEzMyJ9LCJ0aXRsZSI6IlVuY292ZXJpbmcgd2F0ZXIgcXVhbGl0eSBhbmQgZXZhbHVhdGluZyB2dWxuZXJhYmlsaXRpZXMgb2Ygc21hbGwtc2NhbGUgZmlzaGVyaWVzIGluIENoaWxpa2EgTGFnb29uLCBJbmRpYSIsImFic3RyYWN0IjoiU21hbGwtc2NhbGUgZmlzaGVyaWVzIChTU0ZzKSBwbGF5IGEgc3Ryb25nIHJvbGUgaW4gc3VzdGFpbmluZyBtaWxsaW9ucyBvZiBsaXZlbGlob29kcywgZm9vZCBzZWN1cml0eSwgbnV0cml0aW9uLCBhbmQgaW5jb21lIGdsb2JhbGx5IGJ1dCB0aGUgZmlzaGVycyBlbmdhZ2VkIGluIHRoaXMgc2VjdG9yIHNpbXVsdGFuZW91c2x5IGV4cGVyaWVuY2UgaGlnaCBsZXZlbHMgb2YgdnVsbmVyYWJpbGl0eSBhbmQgcHJvY2Vzc2VzIG9mIG1hcmdpbmFsaXNhdGlvbi4gU2V2ZXJhbCBmYWN0b3JzIGFyZSBhdHRyaWJ1dGVkIHRvIHRoZSBtdWx0aWRpbWVuc2lvbmFsIHZ1bG5lcmFiaWxpdGllcyB0aGUgc21hbGwtc2NhbGUgZmlzaGVycyBleHBlcmllbmNlLCBzcGFubmluZyBib3RoIG5hdHVyYWwgKGUuZy4sIG5hdHVyYWwgZGlzYXN0ZXJzLCBlY29zeXN0ZW0gY2hhbmdlKSBhbmQgYW50aHJvcG9nZW5pYyAoZS5nLiwgcG9saWN5IGNoYW5nZSwgaHlkcm9sb2dpY2FsIGludGVydmVudGlvbnMsIGFxdWFjdWx0dXJlKSBwcmVzc3VyZXMuIFdoaWxlIHRoZXJlIGlzIG11Y2ggbGl0ZXJhdHVyZSBvbiB2YXJpb3VzIG5hdHVyYWwgYW5kIGh1bWFuIGRyaXZlcnMgb2YgdnVsbmVyYWJpbGl0eSBpbiBzbWFsbC1zY2FsZSBmaXNoZXJ5IGNvbW11bml0aWVzLCBhbiBhYnNlbmNlIG9mIHJlc2VhcmNoIGNvbm5lY3RpbmcgdnVsbmVyYWJpbGl0eSB3aXRoIHdhdGVyIHF1YWxpdHkgaXMgZXZpZGVudC4gRmlzaGVyIGNvbW11bml0aWVzIG9mdGVuIHRhbGsgYWJvdXQgZmlzaCBpbiByZWxhdGlvbiB0byB0aGUgaGVhbHRoIG9mIHRoZWlyIGFxdWF0aWMgaGFiaXRhdHMgd2hlcmVpbiB3YXRlciBxdWFsaXR5IGlzIHNlZW4gYXMgYSBrZXkgcGFyYW1ldGVyLiBUaGUgbGluayBiZXR3ZWVuIGhlYWx0aHkgZmlzaCBhbmQgZ29vZCBxdWFsaXR5IHdhdGVyIGhhcyBzaWduaWZpY2FudCBpbXBsaWNhdGlvbnMgZm9yIHN0cm9uZyBhbmQgdmlhYmxlIGZpc2hpbmcgY29tbXVuaXRpZXMuIFRoaXMgcGFwZXIgZXhhbWluZXMgdGhlc2UgbGlua3MgZnVydGhlciBieSBmb2N1c2luZyBvbiB0aGUgbmF0dXJlIG9mIHZ1bG5lcmFiaWxpdGllcyBjYXVzZWQgYnkgd2F0ZXIgcXVhbGl0eSBjaGFuZ2VzIGluIHRoZSBzbWFsbC1zY2FsZSBmaXNoZXJ5IHN5c3RlbSBvZiBDaGlsaWthIExhZ29vbiBpbiBJbmRpYS4gV2UgdW5kZXJ0YWtlIGRldGFpbGVkIGFuYWx5c2lzIG9mIHRoZSBpbnZhc2l2ZSBzaHJpbXAgYXF1YWN1bHR1cmUgYWN0aXZpdGllcyBhbmQgaHlkcm9sb2dpY2FsIGludGVydmVudGlvbnMgZm9yIG9wZW5pbmcgb2YgYSBsYWdvb24gaW5sZXQgd2l0aCB0aGUgQmF5IG9mIEJlbmdhbCBhcyB0d28gZG9taW5hbnQgZHJpdmVycyBhZHZlcnNlbHkgaW1wYWN0aW5nIHdhdGVyIHF1YWxpdHkgYW5kIGluY3JlYXNpbmcgdnVsbmVyYWJpbGl0aWVzIG9mIHRoZSBlbnRpcmUgc21hbGwtc2NhbGUgZmlzaGVyaWVzIHNvY2lhbC1lY29sb2dpY2FsIHN5c3RlbS4gT3VyIGFuYWx5c2lzIHN1Z2dlc3RzIHRoYXQgdGhlcmUgYXJlIHN0cm9uZyBpbnRlcmNvbm5lY3Rpb25zIGJldHdlZW4gY2hhbmdlcyBpbiB3YXRlciBxdWFsaXR5IGFuZCB0aGUgbGV2ZWxzIG9mIHZ1bG5lcmFiaWxpdGllcyBpbiB0aGUgU1NGcyBvZiBDaGlsaWthIExhZ29vbi4gUG9sbHV0YW50cyBzdWNoIGFzIHBlc3RpY2lkZXMsIGFuZCBvcmdhbmljIGNvbXBvdW5kcyBhY2N1bXVsYXRlIGluIGZpc2ggdGlzc3VlcyBhbmQgYWZmZWN0IHRoZWlyIGdyb3d0aCwgcmVwcm9kdWN0aW9uLCBhbmQgb3ZlcmFsbCBoZWFsdGguIFRoaXMgbGVkIHRvIGRlY2xpbmVzIGluIGZpc2ggcG9wdWxhdGlvbnMsIG1ha2luZyBpdCBtb3JlIGRpZmZpY3VsdCBmb3IgZmlzaGVycyB0byBtYWtlIGEgbGl2aW5nLiBJbiBhZGRpdGlvbiB0byBkaXJlY3QgaW1wYWN0cyBvbiBmaXNoIHBvcHVsYXRpb25zLCBwb29yIHdhdGVyIHF1YWxpdHkgYWxzbyBoYXMgaW5kaXJlY3QgZWZmZWN0cyBvbiB0aGUgc29jaWFsIGFuZCBlY29ub21pYyB2dWxuZXJhYmlsaXR5IG9mIFNTRnMuIEZvciBleGFtcGxlLCBjb250YW1pbmF0aW9uIG9mIHdhdGVyIHNvdXJjZXMgbGVkIHRvIHRoZSByZWR1Y2VkIG51bWJlciBvZiBmaXNoIHNwZWNpZXMgcmVkdWNpbmcgdGhlIGFtb3VudCBvZiB0aW1lIGZpc2hlcnMgY2FuIHNwZW5kIG9uIGZpc2hpbmcgYWN0aXZpdGllcy4gVGhpcyBhbHNvIGFmZmVjdGVkIHRoZSBtYXJrZXRhYmlsaXR5IG9mIGZpc2ggcHJvZHVjdHMsIHJlZHVjZWQgaW5jb21lIGFuZCBpbmNyZWFzZWQgcG92ZXJ0eS4gVG8gZnVsbHkgdW5kZXJzdGFuZCB0aGUgaW50ZXJjb25uZWN0aW9ucyBiZXR3ZWVuIHdhdGVyIHF1YWxpdHkgYW5kIHZ1bG5lcmFiaWxpdHkgaW4gU1NGcyBpbiBDaGlsaWthIExhZ29vbiwgaXQgaXMgaW1wb3J0YW50IHRvIGNvbnNpZGVyIGJvdGggZW52aXJvbm1lbnRhbCBhbmQgc29jaWFsIGZhY3RvcnMsIGFzIHdlbGwgYXMgdGhlIGNvbXBsZXggZmVlZGJhY2sgbG9vcHMgYmV0d2VlbiB0aGVzZSBmYWN0b3JzLiBUaGUgc3R1ZHkgaGVscHMgaW4gYnJpZGdpbmcgYSBjcnVjaWFsIGdhcCBpbiBvdXIgdW5kZXJzdGFuZGluZyBvZiB0aGUgcm9sZSBvZiB3YXRlciBxdWFsaXR5IGluIHZ1bG5lcmFiaWxpdHkgYW5hbHlzaXMgd2l0aGluIHJlc291cmNlIGRlcGVuZGVudCBjb21tdW5pdGllcy4gV2UgY29uY2x1ZGUgd2l0aCBrZXkgaW5zaWdodHMgb24gcG9zc2libGUgY29waW5nIHJlc3BvbnNlcyBhbmQgYWRhcHRpdmUgY2FwYWNpdHkgbmVjZXNzYXJ5IGZvciB0aGUgc21hbGwtc2NhbGUgZmlzaGVyaWVzIGNvbW11bml0aWVzIHRvIHRyYW5zaXRpb24gdG93YXJkIHZpYWJpbGl0eS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMzM4OS9mbWFycy4yMDIzLjEwODcyOTYiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vd3d3LmZyb250aWVyc2luLm9yZy9hcnRpY2xlcy8xMC4zMzg5L2ZtYXJzLjIwMjMuMTA4NzI5Ni9wZGYiLCJpc3N1ZWREYXRlIjoiMjAyMy0wNi0yOSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZyb250aWVycyBpbiBNYXJpbmUgU2NpZW5jZSIsInZvbHVtZSI6MTAsImltcGFjdEZhY3RvciI6IjQuMjgifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRnJvbnRpZXJzIE1lZGlhIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo1MSwiY2l0ZWRCeUNvdW50Ijo1fX0sInF1b3RlIjoiV2F0ZXIgcXVhbGl0eSBvZiBDaGlsaWthIExhZ29vbiBoYXMgZGVncmFkZWQgZHVlIHRvIGFudGhyb3BvZ2VuaWMgaW50ZXJ2ZW50aW9ucyBhbmQgbmF0dXJhbCBmb3JjZXMgb2YgY2hhbmdlLiBWYXJpb3VzIGFjdGl2aXRpZXMgaW4gdGhlIENoaWxpa2EgTGFnb29uLCBzdWNoIGFzIGluZHVzdHJpYWwgd2FzdGV3YXRlciBkaXNwb3NhbCwgc2V3YWdlIGR1bXBpbmcsIGFxdWFjdWx0dXJlLCBoeWRyb2xvZ2ljYWwgaW50ZXJ2ZW50aW9ucywgYW5kIGN5Y2xvbmVzLCBhcmUgcHJvZHVjaW5nIHNhbGluaXR5IGNoYW5nZXMsIHNlZGltZW50IGRlcG9zaXRpb24sIG51dHJpZW50IGVucmljaG1lbnQsIGV1dHJvcGhpY2F0aW9uLCBhbmQgZGVhZCB6b25lcywgYXMgc3RhdGVkLiBGaXNoIHN1cnZpdmFsIGlzIGF0IHJpc2sgYXMgYSByZXN1bHQgb2YgdGhlIHNpZ25pZmljYW50IHJlZHVjdGlvbiBpbiBlbnZpcm9ubWVudGFsIGNpcmN1bXN0YW5jZXMsIHdoaWNoIGRpbWluaXNoZXMgZmlzaGVybWVu4oCZcyBpbmNvbWUgYW5kIHB1c2hlcyB0aGVtIG91dCBvZiBzb2NpZXR5LiBUaGVzZSBjYXVzZXMgYWZmZWN0aW5nIHRoZSB3YXRlciBxdWFsaXR5IG9mIHRoZSBMYWdvb24gZGVwaWN0IG1hbnkgZmFjZXMgb2YgdnVsbmVyYWJpbGl0eSBpbiBTU0YgY29tbXVuaXRpZXMuIEFzIHRoZSBDaGlsaWthIExhZ29vbiBlbnZpcm9ubWVudCBpcyBleHBlY3RlZCB0byB1bmRlcmdvIGluY3JlYXNpbmdseSBkcmFzdGljIGNoYW5nZXMgKHN1Y2ggYXMgY2F0Y2ggY29tcG9zaXRpb24sIGNhdGNoIGNhcGFjaXR5LCBiaW9kaXZlcnNpdHkgdmFyaWF0aW9ucywgd2F0ZXIgcXVhbGl0eSwgYW5kIGZpc2hlcnkgcmV2ZW51ZXMpLCBpbnRlcm5hdGlvbmFsIHBvbGljeSBoYXMgYmVjb21lIG1vcmUgaW50ZXJlc3RlZCBpbiBhZGFwdGF0aW9uIG1lYXN1cmVzLiBHaXZlbiBob3cgZGl2ZXJzZSBTU0YgZWNvc3lzdGVtcyBhcmUgZXhwZWN0ZWQgdG8gYmUgYmV0d2VlbiBjb250aW5lbnRzLCB0aGVyZSBpcyBubyDigJxvbmUgc2l6ZSBmaXRzIGFsbOKAnSBhcHByb2FjaC4gVGhlIHBhcGVyIG91dGxpbmVkIHNldmVyYWwgY29waW5nIGFuZCBhZGFwdGF0aW9uIG1lY2hhbmlzbXMgdG8gaW1wcm92ZSB3YXRlciBxdWFsaXR5IGFuZCBjb250aW51ZSBmaXNoaW5nIGluIGEgc3VzdGFpbmFibGUgd2F5LiBBZGFwdGF0aW9uIHN0cmF0ZWdpZXMgY2FuIG9jY2FzaW9uYWxseSBoYXZlIHVuZm9yZXNlZW4gY29uc2VxdWVuY2VzIGZvciBmaXNoZXJpZXMgYW5kIGZpc2hpbmcgY29tbXVuaXRpZXMuIEFueSB1bmV4cGVjdGVkIGNvbnNlcXVlbmNlcyBvZiBhZGFwdGF0aW9uIGNhbiBiZSBtaXRpZ2F0ZWQgdGhyb3VnaCBsb25nLXRlcm0gcGxhbm5pbmcgYW5kIGlkZW50aWZpY2F0aW9uIG9mIGZ1dHVyZSBvcHRpb25zIHRocm91Z2ggc2NlbmFyaW8gYW5hbHlzaXMuIFRoZSBmaW5kaW5ncyBvZiB0aGlzIHBhcGVyIGNhbiBiZSB1dGlsaXplZCB0byBkZXZlbG9wIHBvbGljaWVzIHRvIHByb3RlY3Qgd2F0ZXIgcXVhbGl0eSBhbmQgU1NGcyBpbiBhIHZhcmlldHkgb2YgY29hc3RhbCBsYWdvb24gc2V0dGluZ3MuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjIyLjI4NTAzNjA4NzAzNjEzMywibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ5MzA1MzQzNjI3OTMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjIzLjc3MzUxOTUxNTk5MTIxLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDgzMzY0MTA1MjI0NjEsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjI1LjI1NTU4NDcxNjc5Njg3NSwibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ4OTU1OTE3MzU4Mzk4NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjYuNzQ0MDY4MTQ1NzUxOTUzLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNTA2ODA1NDE5OTIxODc1LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyOC4yMjYxMzMzNDY1NTc2MTcsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40OTA0NjMyNTY4MzU5NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjkuNzE0NjE2Nzc1NTEyNjk1LCJsZWZ0Ijo4LjMzMTU5MjU1OTgxNDQ1Mywid2lkdGgiOjQwLjQ5NDA5ODY2MzMzMDA4LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozMS4xOTY2ODM4ODM2NjY5OTIsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40OTQzOTIzOTUwMTk1MywiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MzIuNjg1MTY1NDA1MjczNDQsImxlZnQiOjguMzMxNTk2Mzc0NTExNzE5LCJ3aWR0aCI6NDAuNDg3NDQ1ODMxMjk4ODMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM0LjE2NzIzMjUxMzQyNzczNCwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40OTc3NDU1MTM5MTYwMTYsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM1LjY1NTcxNTk0MjM4MjgxLCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ5ODYzODE1MzA3NjE3LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozNy4xMzc3ODMwNTA1MzcxMSwibGVmdCI6MTAuNjg1MTg2Mzg2MTA4Mzk4LCJ3aWR0aCI6MzguMTQxMTA1NjUxODU1NDcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM4LjYyNjI2NjQ3OTQ5MjE5LCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4NzE1MjA5OTYwOTM3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDAuMTA4MzMzNTg3NjQ2NDg0LCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4OTg1NjcxOTk3MDcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQxLjU5NjgxMzIwMTkwNDMsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDg4NjY2NTM0NDIzODMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQzLjA3ODg4MDMxMDA1ODU5NCwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40ODE4NjExMTQ1MDE5NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDQuNTY3MzYzNzM5MDEzNjcsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDgzMDU4OTI5NDQzMzYsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQ2LjA0OTQzMDg0NzE2Nzk3LCJsZWZ0Ijo4LjMzMzEwODkwMTk3NzUzOSwid2lkdGgiOjQwLjUwMDc0NzY4MDY2NDA2LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo0Ny41Mzc5MTQyNzYxMjMwNSwibGVmdCI6OC4zMzMxMDg5MDE5Nzc1MzksIndpZHRoIjo0MC40OTU2MDU0Njg3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDkuMDE5OTgxMzg0Mjc3MzQ0LCJsZWZ0Ijo4LjMzMzEwNjA0MDk1NDU5LCJ3aWR0aCI6NDAuNTA2ODE2ODY0MDEzNjcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUwLjUwODQ2NDgxMzIzMjQyLCJsZWZ0Ijo4LjMzMzEwNjA0MDk1NDU5LCJ3aWR0aCI6NDAuNDg4NTA2MzE3MTM4NjcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUxLjk5MDUyODEwNjY4OTQ1LCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDk3NzI2NDQwNDI5NjksImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUzLjQ3OTAxMTUzNTY0NDUzLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6MzMuMTU5NDQ2NzE2MzA4NTk0LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo1NC45NjEwNzg2NDM3OTg4MywibGVmdCI6MTAuNjgzNjcwMDQzOTQ1MzEyLCJ3aWR0aCI6MzguMTM2NTk2Njc5Njg3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NTYuNDQ5NTYyMDcyNzUzOTA2LCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDg3MTQ0NDcwMjE0ODQ0LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEzIn19LHsib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRvbSIsImZhbWlseSI6IkppbGJlcnQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUmFvdWzigJBNYXJpZSIsImZhbWlseSI6IkNvdXR1cmUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQnJpYW4iLCJmYW1pbHkiOiJIdXNlciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLYWxldmkiLCJmYW1pbHkiOiJTYWxvbmVuIn19XSwiaWRzIjp7Imlzc24iOlsiMDAxOC04MTU4IiwiMTU3My01MTE3Il0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDA3L3MxMDc1MC0wMjAtMDQ0NTcteCIsIm1hZyI6IjMxMDA0NTkwNTAiLCJvcGVuYWxleCI6IlczMTAwNDU5MDUwIn0sInRpdGxlIjoiUHJlZmFjZTogUmVzdG9yYXRpb24gb2YgZXV0cm9waGljIGxha2VzOiBjdXJyZW50IHByYWN0aWNlcyBhbmQgZnV0dXJlIGNoYWxsZW5nZXMiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAwNy9zMTA3NTAtMDIwLTA0NDU3LXgiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vbGluay5zcHJpbmdlci5jb20vY29udGVudC9wZGYvMTAuMTAwNy9zMTA3NTAtMDIwLTA0NDU3LXgucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjAtMTEtMTYiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA3IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJIeWRyb2Jpb2xvZ2lhIiwiaXNzdWUiOjIxLCJ2b2x1bWUiOjg0NywiaW1wYWN0RmFjdG9yIjoiMy4wMCJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjQzNDMsImxhc3RQYWdlIjo0MzU3fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlNwcmluZ2VyIFNjaWVuY2UrQnVzaW5lc3MgTWVkaWEifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjk1LCJjaXRlZEJ5Q291bnQiOjU4fX0sInF1b3RlIjoiU2ltaWxhciBkZWdyYWRhdGlvbiBvZiBmcmVzaHdhdGVyIGVjb3N5c3RlbXMgaXMgb2JzZXJ2ZWQgd29ybGR3aWRlLCB3aXRoIHRoZSB0aW1lbGluZSBvZiBodW1hbiBwcmVzc3VyZSBkaWN0YXRlZCBieSB0cmVuZHMgaW4gdXJiYW5pemF0aW9uIGFuZCBpbnRlbnNpdmUgYWdyaWN1bHR1cmUgaW4gZGlmZmVyZW50IGdlb2dyYXBoaWMgcmVnaW9ucyAoZS5nLiwgVmVubmVtbyBldCBhbC4sIDIwMDkpLiBUaGUgcHJpbmNpcGFsIGRyaXZlciBvZiBlY29zeXN0ZW0gc3RyZXNzIGluIG1vc3QgYXF1YXRpYyBzeXN0ZW1zIGlzIGV1dHJvcGhpY2F0aW9uLCB0aGUgZXhjZXNzIGdyb3d0aCBvZiBhbGdhZSBkcml2ZW4gYnkgYW50aHJvcG9nZW5pYyBpbnB1dHMgb2YgdGhlIG1hY3JvbnV0cmllbnRzIG5pdHJvZ2VuIChOKSBhbmQgcGhvc3Bob3J1cyAoUCkgKFNtaXRoLCAyMDAzOyBCaXJrIGV0IGFsLiwgMjAyMCkuIE51dHJpZW50cyBhcmUgZW1pdHRlZCBmcm9tIGJvdGggcG9pbnQgYW5kIGRpZmZ1c2Ugc291cmNlcyBpbiB0aGUgaHVtYW4gZW52aXJvbm1lbnQgYW5kIGVudGVyIGFxdWF0aWMgc3lzdGVtcyB2aWEgc3VyZmFjZSBydW5vZmYsIGFzIHdlbGwgYXMgZ3JvdW5kLXdhdGVyIGFuZCBhdG1vc3BoZXJpYyBkZXBvc2l0aW9uIChQYWVybCwgMTk5NykuIEV1dHJvcGhpY2F0aW9uIG9mdGVuIGxlYWRzIHRvIHRoZSBkb21pbmFuY2Ugb2YgcHJpbWFyeSBwcm9kdWN0aW9uIGJ5IGN5YW5vYmFjdGVyaWEsIGluY2x1ZGluZyB0b3hpYyBnZW5lcmEgc3VjaCBhcyBEb2xpY2hvc3Blcm11bSwgQXBoYW5pem9tZW5vbiwgYW5kIE1pY3JvY3lzdGlzIChIdWlzbWFuIGV0IGFsLiwgMjAxOCksIGFzIHdlbGwgYXMgb3h5Z2VuIGRlcGxldGlvbiBhbmQgaGFiaXRhdCBkZWdyYWRhdGlvbiBpbiBsYWtlcyAoQXJlbmQgZXQgYWwuLCAyMDExKS4gQmVjYXVzZSBsYWtlcyBwcm92aWRlIGEgcmFuZ2Ugb2YgY3J1Y2lhbCBlY29zeXN0ZW0gc2VydmljZXMgKFdpbHNvbiAmIENhcnBlbnRlciwgMTk5OSksIHRoZXJlIGlzIGEgZ3Jvd2luZyBuZWVkIGZvciBtYW5hZ2VtZW50IGFwcHJvYWNoZXMgdGhhdCByZXN0b3JlIHdhdGVyIHF1YWxpdHkgaW4gZnJlc2h3YXRlciBzeXN0ZW1zLiBXaXRoaW4gdGhlIGZyYW1ld29yayBvZiBVbml0ZWQgTmF0aW9ucyBTdXN0YWluYWJsZSBEZXZlbG9wLW1lbnQgR29hbCA2IChDbGVhbiBXYXRlciBhbmQgU2FuaXRhdGlvbiksIHNldmVyYWwgZGlzdGluY3QgdGFyZ2V0cyByZWxhdGUgdG8gdGhlIGltcHJvdmVtZW50IG9mIHdhdGVyIHF1YWxpdHkgYW5kIHByb3RlY3Rpb24gb2YgYXF1YXRpYyBlY29zeXN0ZW1zIGJ5IDIwMzAgKFVuaXRlZCBOYXRpb25zLCAyMDE1KS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyOC4yOTg4NjQzNjQ2MjQwMjMsImxlZnQiOjguNTQ5MjA1NzgwMDI5Mjk3LCJ3aWR0aCI6MzkuMzcxNDk0MjkzMjEyODksImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MjkuOTkxMTM0NjQzNTU0Njg4LCJsZWZ0Ijo4LjU0OTIwNTc4MDAyOTI5Nywid2lkdGgiOjM5LjM2NTExOTkzNDA4MjAzLCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMxLjY4MzQwNjgyOTgzMzk4NCwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNjk2NjMyMzg1MjUzOSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozMy4zNjc5NzMzMjc2MzY3MiwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNzU2NzkwMTYxMTMyOCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNS4wNjAyNDkzMjg2MTMyOCwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNzA5NjQwNTAyOTI5NywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNi43NTI1MTc3MDAxOTUzMSwibGVmdCI6OC41NDkyMDg2NDEwNTIyNDYsIndpZHRoIjozOS4zNzY5NjA3NTQzOTQ1MywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC40MzcwODgwMTI2OTUzMSwibGVmdCI6OC41NDkyMDg2NDEwNTIyNDYsIndpZHRoIjozOS4zNjY1Njk1MTkwNDI5NywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0MC4xMjkzNTYzODQyNzczNDQsImxlZnQiOjguNTQ5MjA4NjQxMDUyMjQ2LCJ3aWR0aCI6MzkuMzY0Mzc5ODgyODEyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0MS44MjE2MjQ3NTU4NTkzNzUsImxlZnQiOjguNTQ5MjA4NjQxMDUyMjQ2LCJ3aWR0aCI6MzkuMzY1NjczMDY1MTg1NTUsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDMuNTA2MTk1MDY4MzU5Mzc1LCJsZWZ0Ijo4LjU0OTM4OTgzOTE3MjM2Mywid2lkdGgiOjM5LjM3MDM5MTg0NTcwMzEyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0NS4xOTg0NjM0Mzk5NDE0MDYsImxlZnQiOjguNTQ5NTcyOTQ0NjQxMTEzLCJ3aWR0aCI6MzkuMzY1ODQ0NzI2NTYyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ni44OTA3MzU2MjYyMjA3LCJsZWZ0Ijo4LjU0OTU3Mjk0NDY0MTExMywid2lkdGgiOjM5LjM3Mjk1NTMyMjI2NTYyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0OC41NzUzMDIxMjQwMjM0NCwibGVmdCI6OC41NDk1NzI5NDQ2NDExMTMsIndpZHRoIjozOS4zNzI0MjEyNjQ2NDg0NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1MC4yNjc1NzgxMjUsImxlZnQiOjguNTQ5NTc2NzU5MzM4Mzc5LCJ3aWR0aCI6MzkuMzcxMTI0MjY3NTc4MTI1LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjUxLjk1OTg0MjY4MTg4NDc2NiwibGVmdCI6OC41NDk1NzY3NTkzMzgzNzksIndpZHRoIjozOS4zNjc4MzU5OTg1MzUxNTYsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTMuNTc0MTE5NTY3ODcxMDk0LCJsZWZ0Ijo4LjU0OTU3Njc1OTMzODM3OSwid2lkdGgiOjM5LjM3MDc3NzEzMDEyNjk1LCJoZWlnaHQiOjEuMzM2ODk1NTg1MDYwMTE5Nn19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjU1LjI2NjM5MTc1NDE1MDM5LCJsZWZ0Ijo4LjU0OTc2MDgxODQ4MTQ0NSwid2lkdGgiOjM5LjM2NjU4MDk2MzEzNDc2NiwiaGVpZ2h0IjoxLjMzNjg5NTU4NTA2MDExOTZ9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1Ny4wMjg5NTM1NTIyNDYwOTQsImxlZnQiOjguNTQ5OTQ0ODc3NjI0NTEyLCJ3aWR0aCI6MzkuMzYyMDI2MjE0NTk5NjEsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTguNzEzNTIzODY0NzQ2MDk0LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjkuMjk5OTgyMDcwOTIyODUyLCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjYwLjQwNTc5MjIzNjMyODEyNSwibGVmdCI6MTAuNjIyNDU1NTk2OTIzODI4LCJ3aWR0aCI6MzcuMjg3ODY4NDk5NzU1ODYsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NjIuMDk4MDYwNjA3OTEwMTU2LCJsZWZ0Ijo4LjU0OTk0MzkyMzk1MDE5NSwid2lkdGgiOjM5LjM3MzMyNTM0NzkwMDM5LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjYzLjc4MjYzMDkyMDQxMDE1NiwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzMxMjY5ODM2NDI1OCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2NS40NzQ5MDY5MjEzODY3MiwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNjUxMTk5MzQwODIwMywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2Ny4xNjcxNzUyOTI5Njg3NSwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNjk0ODAxMzMwNTY2NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2OC43ODE0NDgzNjQyNTc4MSwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzczMzg0MDk0MjM4MywiaGVpZ2h0IjoxLjMzNjg5MTUzMTk0NDI3NX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjcwLjU0NDAxMzk3NzA1MDc4LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjM5LjM2OTMwODQ3MTY3OTY5LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjcyLjIzNjI4OTk3ODAyNzM0LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjM5LjM3MjAzMjE2NTUyNzM0NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo3My45MjA4NDUwMzE3MzgyOCwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzEzMDczNzMwNDY4NzUsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNDM0NCJ9fV0%3D. Himachal Pradesh and Manipur have state wetland rules aligned with NLCP, focusing on Rewalsar and Loktak Lakes, yet legacy phosphorus recycling under warming stratification perpetuates HABs (Kumar & Mahajan, 2020; Nõges et al., 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlBhd2FuIiwiZmFtaWx5IjoiS3VtYXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW1icmlzaCBLdW1hciIsImZhbWlseSI6Ik1haGFqYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyNTIzLTM5NjMiLCIyNTIzLTM5NzEiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMDcvczQyNDUyLTAyMC0zMDgyLTgiLCJtYWciOiIzMDM1OTYwMjI4Iiwib3BlbmFsZXgiOiJXMzAzNTk2MDIyOCJ9LCJ0aXRsZSI6IlRyb3BoaWMgc3RhdHVzIGFuZCBpdHMgcmVndWxhdGluZyBmYWN0b3IgZGV0ZXJtaW5hdGlvbiBhdCB0aGUgUmV3YWxzYXIgTGFrZSwgbm9ydGh3ZXN0IEhpbWFsYXlhIChIUCksIEluZGlhLCBiYXNlZCBvbiBzZWxlY3RlZCBwYXJhbWV0ZXJzIGFuZCBtdWx0aXZhcmlhdGUgc3RhdGlzdGljYWwgYW5hbHlzaXMiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAwNy9zNDI0NTItMDIwLTMwODItOCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9saW5rLnNwcmluZ2VyLmNvbS9jb250ZW50L3BkZi8xMC4xMDA3L3M0MjQ1Mi0wMjAtMzA4Mi04LnBkZiIsImlzc3VlZERhdGUiOiIyMDIwLTA2LTIyIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiU04gQXBwbGllZCBTY2llbmNlcyIsImlzc3VlIjo3LCJ2b2x1bWUiOjIsImltcGFjdEZhY3RvciI6IjcuMzUifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiU3ByaW5nZXIgTmF0dXJlIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo0MCwiY2l0ZWRCeUNvdW50IjoxOX19LCJxdW90ZSI6IkNoYW5naW5nIHF1YWxpdHkgaGFzIGNyZWF0ZWQgdGhlIHdhdGVyIHNjYXJjaXR5IGFuZCAgYSBodWdlIG1hc3Mgb2YgcG9wdWxhdGlvbiB0aHVzIGJlZW4gZGVwcml2ZWQgZnJvbSB1c2luZyAgdGhlIGZyZXNod2F0ZXIgZm9yIHZhcmlvdXMgcHVycG9zZXMgWzMsIDRdLiBIb3dldmVyLCB0aGUgIG9ibm94aW91cyBtYXRlcmlhbHMgdGhhdCBzdGF5IGluIHdhdGVyIGJvZGllcyBmb3IgYSBsb25nLXRpbWUgcGVyaW9kIGJyaW5nIGNoYW5nZXMgaW4gdGhlIHBoeXNpb2NoZW1pY2FsIGNoYXItYWN0ZXJpc3RpY3MgYXMgd2VsbCBhcyB0aGUgbmF0dXJhbCBzY2VuYXJpbyBvZiB0aGUgbGFrZXMgWzVdLiAgVGhlIEhpbWFsYXlhbiBsYWtlcyBhcmUgdW5kZXIgc3RyZXNzIGR1ZSB0byBtaXNtYW5hZ2VkIGh1bWFuIGFjdGl2aXRpZXMsIHdoaWNoIGluY3JlYXNlIG51dHJpZW50cyBhbmQgcG9sbHV0aW9uIGluIHRlcm1zIG9mIGhlYXZ5IG1ldGFscyB0b3hpY2l0eSBhbmQgYmFjdGVyaWFsIGNvbnRhbWluYXRpb24gIFs24oCTOF0uIFRoZSBtYW5hZ2VtZW50IGFuZCBwcmVzZXJ2YXRpb24gb2YgdGhlIGxha2VzIGF0ICBwcmVzZW50IGhhdmUgYmVjb21lIGEgZ3JlYXQgY2hhbGxlbmdlIGZvciBlbnZpcm9ubWVudGFsLWlzdHMgYW5kIG1hbmFnZXJzIFs1LCA5XS4gVGhlIGV1dHJvcGhpY2F0aW9uIGlzIGFsc28gYSBtYWluIHByb2JsZW0gb2YgdGhlICBmcmVzaHdhdGVyIGxha2VzLCBhbmQgaXQgaXMgc3BlY2lmaWVkIGZvciB0aGUgbGFrZXMgaGF2aW5nICBudXRyaWVudCBlbnJpY2htZW50LiBJdCBjYXVzZXMgZXhjZXNzaXZlIGdyb3d0aCBvZiBhbGdhZSAgYW5kIGlzIGNvbnNpZGVyZWQgdG8gYmUgcmVzcG9uc2libGUgZm9yIGRlZ3JhZGluZyB0aGUgIHF1YWxpdGF0aXZlIG5hdHVyZSBvZiB0aGUgbGFrZXMgWzUsIDEwXS4gVGhlIGV1dHJvcGhpY2F0aW9uICBpcyBmb2xsb3dlZCBieSB0aGUgYWxnYWwgYmxvb20sIHdoaWNoIGdlbmVyYXRlcyB0b3hpbiwgIHdvcnNlbnMgd2F0ZXIgcXVhbGl0eSwgZGVwbGV0ZXMgZGlzc29sdmVkIG94eWdlbiAoRE8pICogUGF3YW4gS3VtYXIsIHBhd2FuLmV2c0BnbWFpbC5jb20gfCAxRGVwYXJ0bWVudCBvZiBFbnZpcm9ubWVudGFsIFNjaWVuY2VzLCBDZW50cmFsIFVuaXZlcnNpdHkgb2YgSGltYWNoYWwgUHJhZGVzaCwgRGhhcmFtc2hhbGEgMTc2MjE1LCBJbmRpYS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2OC41Nzg0NjA2OTMzNTkzOCwibGVmdCI6OC41NzE0MTg3NjIyMDcwMzEsIndpZHRoIjo0MC4zMjc1MTQ2NDg0Mzc1LCJoZWlnaHQiOjEuNTE5ODU1MjYwODQ4OTk5fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NzAuMTU4OTk2NTgyMDMxMjUsImxlZnQiOjguNTY5NzM4Mzg4MDYxNTIzLCJ3aWR0aCI6NDAuMzU0NzI0ODg0MDMzMiwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjcxLjczOTU0NzcyOTQ5MjE5LCJsZWZ0Ijo4LjU2OTczODM4ODA2MTUyMywid2lkdGgiOjQwLjM0OTY5MzI5ODMzOTg0NCwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjczLjMyMDA5MTI0NzU1ODYsImxlZnQiOjguNTY4MDU5OTIxMjY0NjQ4LCJ3aWR0aCI6NDAuMDIwMDg4MTk1ODAwNzgsImhlaWdodCI6MS41MTk4NTUyNjA4NDg5OTl9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo3NC45MDA2NDIzOTUwMTk1MywibGVmdCI6OC41NjgwNTk5MjEyNjQ2NDgsIndpZHRoIjozOS45OTk5Mjc1MjA3NTE5NSwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjc2LjQ4MTE4NTkxMzA4NTk0LCJsZWZ0Ijo4LjU2ODA1NzA2MDI0MTcsIndpZHRoIjo0MC4zNTcwNzQ3Mzc1NDg4MywiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjc4LjA2MTcyOTQzMTE1MjM0LCJsZWZ0Ijo4LjU2OTczNjQ4MDcxMjg5LCJ3aWR0aCI6NDAuMzUzNzEzOTg5MjU3ODEsImhlaWdodCI6MS41MTk4NTUyNjA4NDg5OTl9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2MC42NzU3Mjc4NDQyMzgyOCwibGVmdCI6NTEuNDI3MTY1OTg1MTA3NDIsIndpZHRoIjo0MC4zNzg1NjY3NDE5NDMzNiwiaGVpZ2h0IjoxOC45MDU4NTUxNzg4MzMwMDh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo4MS45MjczMzc2NDY0ODQzOCwibGVmdCI6OC40MDE0OTc4NDA4ODEzNDgsIndpZHRoIjo3My45NTQ2OTY2NTUyNzM0NCwiaGVpZ2h0IjoyLjY3ODc2MjY3NDMzMTY2NX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEifX0seyJvcmlnaW4iOjUsIndvcmsiOnsidHlwZSI6OCwic3VidHlwZSI6eyJvbmVvZktpbmQiOiJhcnRpY2xlVHlwZSIsImFydGljbGVUeXBlIjoxfSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiVGlpbmEiLCJmYW1pbHkiOiJOw7VnZXMifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTmFzaW1lIiwiZmFtaWx5IjoiSmFuYXRpYW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUmVldCIsImZhbWlseSI6IkxhdWdhc3RlIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlBlZXRlciIsImZhbWlseSI6Ik7DtWdlcyJ9fV0sImlkcyI6eyJpc3NuIjpbIjIzNTEtOTg5NCJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMTAxNi9qLmdlY2NvLjIwMjAuZTAxMzY5IiwibWFnIjoiMzEwNTQ0NjA5MCIsIm9wZW5hbGV4IjoiVzMxMDU0NDYwOTAiLCJleGEiOiJodHRwczovL3d3dy5zY2llbmNlZGlyZWN0LmNvbS9zY2llbmNlL2FydGljbGUvcGlpL1MyMzUxOTg5NDIwMzA5MTAwIn0sInRpdGxlIjoiUG9zdC1zb3ZpZXQgY2hhbmdlcyBpbiBuaXRyb2dlbiBhbmQgcGhvc3Bob3J1cyBzdG9pY2hpb21ldHJ5IGluIHR3byBsYXJnZSBub24tc3RyYXRpZmllZCBsYWtlcyBhbmQgdGhlIGltcGFjdCBvbiBwaHl0b3BsYW5rdG9uIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjEwMTYvai5nZWNjby4yMDIwLmUwMTM2OSIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjEwMTYvai5nZWNjby4yMDIwLmUwMTM2OSIsImlzc3VlZERhdGUiOiIyMDIwLTExLTIwIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMSIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiR2xvYmFsIEVjb2xvZ3kgYW5kIENvbnNlcnZhdGlvbiIsInZvbHVtZSI6MjQsImltcGFjdEZhY3RvciI6IjMuNTkifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRWxzZXZpZXIgQlYifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjcxLCJjaXRlZEJ5Q291bnQiOjE0fX0sInF1b3RlIjoiUmFuZG9tIEZvcmVzdCBhbmFseXNpcyBpbmRpY2F0ZWQgYW4gaW1wb3J0YW50IHJvbGUgb2YgbGlnaHQgbGltaXRhdGlvbiBpbiBib3RoIGxha2VzLiBDb25zdGFudGx5IGxvd2VyIGxldmVscyBvZiBOIGluIHRoZSBkZWVwZXIgTGFrZSBQZWlwc2kgZmF2b3VyZWQgTjItZml4aW5nIHNwZWNpZXMsIHdoaWNoLCBhcyBhIHBhcmFkb3gsIGJlY2FtZSBQLWxpbWl0ZWQuIEFzIGNsaW1hdGUgd2FybWluZyByZWluZm9yY2VzIGV1dHJvcGhpY2F0aW9uIHBoZW5vbWVuYSBpbiBsYWtlcyBieSBpbmNyZWFzaW5nIGludGVybmFsIG51dHJpZW50IGxvYWRpbmcgYW5kIGZhdm91cmluZyBibG9vbS1mb3JtaW5nIGN5YW5vYmFjdGVyaWEsIG1vcmUgc3RyaW5nZW50IG1lYXN1cmVzIHdvdWxkIGJlIG5lZWRlZCB0byBmdXJ0aGVyIGxpbWl0IG51dHJpZW50IGxvYWRzIChlc3BlY2lhbGx5IHRoYXQgb2YgUCkgdG8gbGFrZXMgdGhyb3VnaCBpbXByb3ZlZCB3YXN0ZXdhdGVyIHRyZWF0bWVudCBhbmQgaW5jcmVhc2VkIGVmZmljaWVuY3kgb2YgZmVydGlsaXNlciBhcHBsaWNhdGlvbi4gTG9uZy10ZXJtIGNoYW5nZXMgaW4gbnV0cmllbnQgbG9hZGluZyBhbmQgc3RvaWNoaW9tZXRyeSBpbiB3YXRlciBib2RpZXMgcmVmbGVjdCBjaGFuZ2VzIGluIGFncmljdWx0dXJhbCBwcmFjdGljZXMgYW5kIHdhc3Rld2F0ZXIgdHJlYXRtZW50LiJ9XQ%3D%3D. These initiatives allocate funds e.g., INR 200 crore for Chilika but suffer from inconsistent monitoring and overlapping jurisdictions between pollution control boards and local bodies.

Top-down approaches falter primarily due to inadequate localization and enforcement mechanisms. The major limitations of existing regulatory frameworks for HAB management are summarized in Table 5. National standards, such as primary water quality criteria (e.g., BOD <3 mg/L, phosphorus <0.1 mg/L for lakes), prove insufficient against HAB thresholds, as evidenced by persistent Microcystis blooms despite compliance claims (Foulon et al., 2019)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IsOJdGllbm5lIiwiZmFtaWx5IjoiRm91bG9uIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFsYWluIE4uIiwiZmFtaWx5IjoiUm91c3NlYXUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiR2xlbm4iLCJmYW1pbHkiOiJCZW5veSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJSZWJlY2NhIEwuIiwiZmFtaWx5IjoiTm9ydGgifX1dLCJpZHMiOnsiaXNzbiI6WyIxMjAxLTMwODAiLCIyNDA4LTk0NDMiXSwiaXNibiI6W10sImRvaSI6IjEwLjIxNjYvd3Fyai4yMDE5LjAxMyIsIm1hZyI6IjI5NzM4NjAyODEiLCJvcGVuYWxleCI6IlcyOTczODYwMjgxIn0sInRpdGxlIjoiQSBnbG9iYWwgc2NhbiBvZiBob3cgdGhlIGlzc3VlIG9mIG51dHJpZW50IGxvYWRpbmcgYW5kIGhhcm1mdWwgYWxnYWwgYmxvb21zIGlzIGJlaW5nIGFkZHJlc3NlZCBieSBnb3Zlcm5tZW50cywgbm9uLWdvdmVybm1lbnRhbCBvcmdhbml6YXRpb25zLCBhbmQgdm9sdW50ZWVycyIsImFic3RyYWN0IjoiQWJzdHJhY3QgSGFybWZ1bCBhbGdhbCBibG9vbXMgKEhBQnMpIGluIGFxdWF0aWMgZWNvc3lzdGVtcyBhcmUgb2YgY29uY2VybiB3b3JsZHdpZGUuIFRoaXMgcmV2aWV3IGRlYWxzIHdpdGggaG93IGp1cmlzZGljdGlvbnMgYXJvdW5kIHRoZSB3b3JsZCBhcmUgYWRkcmVzc2luZyB0aGlzIHdhdGVyIHF1YWxpdHkgaXNzdWUgdG8gaW5mb3JtIHJlY29tbWVuZGF0aW9ucyByZWdhcmRpbmcgbnV0cmllbnQgbG9hZGluZyBhbmQgSEFCcyBpbiBNaXNzaXNxdW9pIEJheS1MYWtlIENoYW1wbGFpbiBhbmQgTGFrZSBNZW1waHJlbWFnb2c7IHRyYW5zYm91bmRhcnkgbGFrZXMgbG9jYXRlZCBpbiB0aGUgVVNBIGFuZCBDYW5hZGEgdGhhdCBzdWZmZXIgZnJvbSBzeW1wdG9tcyBvZiBldXRyb3BoaWNhdGlvbi4gQSBnbG9iYWwgc2NhbiBvZiB0aGUgbGl0ZXJhdHVyZSByZXN1bHRlZCBpbiB0aGUgY29uc2lkZXJhdGlvbiBvZiAxMiBjYXNlIHN0dWRpZXMgb2YgbGFyZ2Ugd2F0ZXIgYm9kaWVzIHdpdGhpbiBsYXJnZSB3YXRlcnNoZWRzLCBleGNsdWRpbmcgaW4tbGFrZSBnZW9lbmdpbmVlcmluZyBhcHByb2FjaGVzLiBBbHRob3VnaCBhbGwgb2YgdGhlIHN5c3RlbXMgZXhwZXJpZW5jZSBleGNlc3NpdmUgbnV0cmllbnQgbG9hZGluZywgdGhleSB2YXJ5IGluIHR3byBrZXkgd2F5czogc291cmNlcyBvZiBudXRyaWVudHMgYW5kIG1hbmlmZXN0YXRpb25zIG9mIGV1dHJvcGhpY2F0aW9uIHJhbmdpbmcgZnJvbSBIQUJzLCB0byBsaW1pdGVkIHJlY3JlYXRpb25hbCB1c2VzLCB0byB0aGUgYWRkaXRpb25hbCBjb21wbGV4aXR5IG9mIGludGVybmFsIGxvYWRpbmdzIGFuZCBmaXNoIGtpbGxzLCB1cCB0byBkcmlua2luZyB3YXRlciBzaHV0ZG93bnMuIFRoZSBjYXNlIHN0dWRpZXMgd2VyZSBhbmFseXplZCB3aXRoIHJlc3BlY3QgdG8gZm91ciBjYXRlZ29yaWVzIG9mIGFwcHJvYWNoZXMsIG5hbWVseTogKGkpIHJlZ3VsYXRvcnk7IChpaSkgaW5jZW50aXZlLWJhc2VkOyAoaWlpKSByaXNrIG1pdGlnYXRpb247IGFuZCAoaXYpIG91dHJlYWNoLCBlbmdhZ2VtZW50LCBhbmQgZWR1Y2F0aW9uYWwuIFdlIGZvdW5kIHRoYXQgdGhlIG1hbmFnZW1lbnQgZnJhbWV3b3JrcyBhcmUgYmFzZWQgb24gaW50ZWdyYXRlZCB3YXRlcnNoZWQgbWFuYWdlbWVudCBwbGFubmluZyBhbmQgbmF0aW9uYWwgc3RhbmRhcmRzLiBOYXRpb25hbCB3YXRlciBxdWFsaXR5IHN0YW5kYXJkcywgaG93ZXZlciwgYXJlIG5vdCBzdHJpbmdlbnQgZW5vdWdoIHRvIHByZXZlbnQgSEFCcy4gT3ZlcmFsbCwgaWRlbnRpZmllZCBjYXNlIHN0dWRpZXMgZGlkIG5vdCBzdWNjZXNzZnVsbHkgcmVtZWRpYXRlIEhBQnMsIHRoZXkgc2ltcGx5IG1hbmFnZWQgdGhlbS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjE2Ni93cXJqLjIwMTkuMDEzIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2l3YXBvbmxpbmUuY29tL3dxcmovYXJ0aWNsZS1wZGYvNTUvMS8xLzcwOTI2MS93cXJqYzA1NTAwMDEucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMTktMDktMTciLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA5IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJXYXRlciBRdWFsaXR5IFJlc2VhcmNoIEpvdXJuYWwiLCJpc3N1ZSI6MSwidm9sdW1lIjo1NSwiaW1wYWN0RmFjdG9yIjoiNy44MyJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjEsImxhc3RQYWdlIjoyM319LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJVV0EgUHVibGlzaGluZyJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6ODQsImNpdGVkQnlDb3VudCI6MjB9fSwicXVvdGUiOiJXZSBwcm9wb3NlIHNvbHV0aW9ucyBhbmQgcGF0aHMgZm9yd2FyZCB0byBhIHdpY2tlZCBwcm9ibGVtIHdoaWNoIHNob3VsZCBwcm92ZSBpbnNpZ2h0ZnVsIGZvciBwb2xpY3ktbWFrZXJzIGFuZCBtYW4tIGFnZXJzIGFsaWtlIHRocm91Z2ggdGhlIGZvbGxvd2luZyBmaW5kaW5ncy4gTmF0aW9uYWwgd2F0ZXIgcXVhbGl0eSBzdGFuZGFyZHMgaGF2ZSBub3QgcHJvdmVkIHN0cmluZ2VudCBlbm91Z2ggdG8gcHJlLSB2ZW50IEhBQnMuIElkZW50aWZpZWQgZnJhbWV3b3JrcyBhcmUgYWxsIGJhc2VkIG9uIGludGVncmF0ZWQgd2F0ZXJzaGVkIG1hbmFnZW1lbnQgYW5kIG5hdGlvbmFsIHN0YW5kYXJkcy4gTm8gYWN0dWFsIHN1Y2Nlc3Mgc3RvcmllcyByZWxhdGVkIHRvIHJlbWVkaWF0aW9uIG9mIEhBQnMgd2VyZSBpZGVudGlmaWVkOyB0aGV5IHdlcmUgc2ltcGx5IG1hbmFnZW1lbnQgc3Rvcmllcy4gVGhlIGVmZmljYWN5IG9mIE5QUyBCTVBzIGlzIHF1ZXN0aW9uZWQgdGhyb3VnaG91dCB0aGUgd29ybGQgZHVlIHRvIGluc3VmZmljaWVudCBldmlkZW5jZSB0byBkZXRlcm1pbmUgd2hldGhlciB0aGV5IGNhbiBtaXRpZ2F0ZSBIQUJzLiBNYXJrZXQtYmFzZWQgc3RyYXRlZ2llcyB0byBvZmZzZXQgcG9sbHV0aW9uIG9yIHByb21vdGUgdGFyZ2V0ZWQgQk1QIGFkb3B0aW9uIGFyZSBiZWNvbWluZyBwb3B1bGFyIGFuZCByZXN1bHRpbmcgaW4gY29zdC1lZmZlY3RpdmUgYW5kIGVmZmljaWVudCBhcHByb2FjaGVzLiBDb3N04oCTYmVuZWZpdCBhbmFseXNlcyBtYXkgaW5jcmVhc2UgdGhlIHVzZSBvZiB0ZWNobmljYWwgc29sLSB1dGlvbnMgb3ZlciBjb21wcmVoZW5zaXZlIGFwcHJvYWNoZXMuIEF0IHRoaXMgcG9pbnQsIHRoZXJlIGlzIG5vdCByZWFsbHkgYSB1bmlxdWUgY2FzZSBzdHVkeSB0aGF0IGNvdWxkIGJlIGFwcGxpZWQgdG8gTGFrZSBDaGFtcGxhaW4gYW5kIExha2UgTWVtcGhyZW1hZ29nLiBTcGVjaWZpYyBlbGVtZW50cyBvZiB0aGUgYXBwcm9hY2hlcyBpZGVudGlmaWVkIGluIHRoaXMgcmV2aWV3IGFuZCB1bmktIGZ5aW5nIHRoZW1lcywgaG93ZXZlciwgaG9sZCBwcm9taXNlcy4gVGhpcyBpcyB3aHkgd2UgYWR2b2NhdGUgZm9yIGNvbmR1Y3RpbmcgYSBzeXN0ZW1hdGljIGxpdGVyYXR1cmUgcmV2aWV3IHdpdGggcmVnYXJkIHRvIHRob3NlIHRoZW1lcyBpbiBvcmRlciB0byBkZXZlbG9wIHJlY29tbWVuLSBkYXRpb25zIHRhaWxvcmVkIHRvIHRoZSBMYWtlIE1lbXBocmVtYWdvZyBhbmQgTWlzc2lzcXVvaSBCYXktTGFrZSBDaGFtcGxhaW4gYXJlYXMuIEF0IGxhc3QsIHdlIHdvdWxkIGxpa2UgdG8gYWRkcmVzcyB0aGUgZWxlcGhhbnQtc2l6ZWQgcXVlcy0gdGlvbiBpbiB0aGUgcm9vbS4gV2lsbCBjbGltYXRlIGNoYW5nZSBpbmNyZWFzZSBIQUJzIGZyZXF1ZW5jeT8gSW50ZXJhY3RpdmUgZWZmZWN0cyBvZiBldXRyb3BoaWNhdGlvbiBhbmQgY2xpbWF0ZSBjaGFuZ2Ugb24gaGFybWZ1bCBjeWFub2JhY3RlcmlhbCBibG9vbXMgYXJlIGNvbXBsZXguICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxNSwiYmJveCI6eyJ0b3AiOjg1LjYwNTUxNDUyNjM2NzE5LCJsZWZ0Ijo0OS43OTQ4NzIyODM5MzU1NSwid2lkdGgiOjM5LjcxODY2MjI2MTk2Mjg5LCJoZWlnaHQiOjEuMzI4NjcxMzM2MTc0MDExMn19LHsicGFnZU51bWJlciI6MTUsImJib3giOnsidG9wIjo4Ny4zOTUzNjI4NTQwMDM5LCJsZWZ0Ijo0OS43OTQ4NzIyODM5MzU1NSwid2lkdGgiOjM5LjcwMjcwOTE5Nzk5ODA1LCJoZWlnaHQiOjEuMzI4NjcxMzM2MTc0MDExMn19LHsicGFnZU51bWJlciI6MTUsImJib3giOnsidG9wIjo4OS4xODUyMTExODE2NDA2MiwibGVmdCI6NDkuNzk0ODcyMjgzOTM1NTUsIndpZHRoIjozOS43MDIwOTUwMzE3MzgyOCwiaGVpZ2h0IjoxLjMyODY3MTMzNjE3NDAxMTJ9fSx7InBhZ2VOdW1iZXIiOjE1LCJiYm94Ijp7InRvcCI6OTAuOTc1MDU5NTA5Mjc3MzQsImxlZnQiOjQ5Ljc5NDg3MjI4MzkzNTU1LCJ3aWR0aCI6MzkuNzA0NjIwMzYxMzI4MTI1LCJoZWlnaHQiOjEuMzI4NjcxMzM2MTc0MDExMn19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjoxNC4wMjU5OTA0ODYxNDUwMiwibGVmdCI6MTAuNTAyMzIzMTUwNjM0NzY2LCJ3aWR0aCI6MzkuNzA1OTU1NTA1MzcxMDk0LCJoZWlnaHQiOjEuMzI4NjY5NDI4ODI1Mzc4NH19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjoxNS44MTU4NDE2NzQ4MDQ2ODgsImxlZnQiOjEwLjUwMjMyMzE1MDYzNDc2Niwid2lkdGgiOjM5LjcwMjI0NzYxOTYyODkwNiwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MTcuNjA1NjkxOTA5NzkwMDQsImxlZnQiOjEwLjUwMjE3NjI4NDc5MDAzOSwid2lkdGgiOjM5LjcwMDkyMzkxOTY3NzczNCwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MTkuMzk1NTQ0MDUyMTI0MDIzLCJsZWZ0IjoxMC41MDIxNzYyODQ3OTAwMzksIndpZHRoIjozOS43MDg5MDA0NTE2NjAxNTYsImhlaWdodCI6MS4zMjg2Njk0Mjg4MjUzNzg0fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjIxLjE4NTM5NDI4NzEwOTM3NSwibGVmdCI6MTAuNTAyMTc2Mjg0NzkwMDM5LCJ3aWR0aCI6MzkuNzEwMDgzMDA3ODEyNSwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MjIuOTc1MjQ0NTIyMDk0NzI3LCJsZWZ0IjoxMC41MDIxNzYyODQ3OTAwMzksIndpZHRoIjozOS43MDQwMjkwODMyNTE5NSwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MjQuNzY1MDk2NjY0NDI4NzEsImxlZnQiOjEwLjUwMjE3NjI4NDc5MDAzOSwid2lkdGgiOjM5LjcwMzg4MDMxMDA1ODU5NCwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MjYuNTU0OTQ4ODA2NzYyNjk1LCJsZWZ0IjoxMC41MDIwMjc1MTE1OTY2OCwid2lkdGgiOjM5LjcwMzg4MDMxMDA1ODU5NCwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MjguMzQ0Nzk5MDQxNzQ4MDQ3LCJsZWZ0IjoxMC41MDIwMjc1MTE1OTY2OCwid2lkdGgiOjM5LjcwMDAzODkwOTkxMjExLCJoZWlnaHQiOjEuMzI4NjY5NDI4ODI1Mzc4NH19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjozMC4xMzQ2NDczNjkzODQ3NjYsImxlZnQiOjEwLjUwMjAyNzUxMTU5NjY4LCJ3aWR0aCI6MzkuNzA0NjE2NTQ2NjMwODYsImhlaWdodCI6MS4zMjg2Njk0Mjg4MjUzNzg0fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjMxLjkyNDQ5NzYwNDM3MDExNywibGVmdCI6MTAuNTAxODc3Nzg0NzI5MDA0LCJ3aWR0aCI6MzkuNzA1Nzg3NjU4NjkxNDA2LCJoZWlnaHQiOjEuMzI4NjY5NDI4ODI1Mzc4NH19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjozMy43MTQzNTE2NTQwNTI3MzQsImxlZnQiOjEwLjUwMTcyOTAxMTUzNTY0NSwid2lkdGgiOjM5LjcxOTI0NTkxMDY0NDUzLCJoZWlnaHQiOjEuMzI4NjY3NDAyMjY3NDU2fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjM1LjUwNDIwMzc5NjM4NjcyLCJsZWZ0IjoxMC41MDE3MjkwMTE1MzU2NDUsIndpZHRoIjozOS43MDI3MDkxOTc5OTgwNSwiaGVpZ2h0IjoxLjMyODY2NzQwMjI2NzQ1Nn19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjozNy4yOTQwNTIxMjQwMjM0NCwibGVmdCI6MTAuNTAxNzI5MDExNTM1NjQ1LCJ3aWR0aCI6MzkuNzA1Nzk5MTAyNzgzMiwiaGVpZ2h0IjoxLjMyODY2NzQwMjI2NzQ1Nn19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjozOS4wODM5MDQyNjYzNTc0MiwibGVmdCI6MTAuNTAxNzI1MTk2ODM4Mzc5LCJ3aWR0aCI6MzkuNzA2NTM1MzM5MzU1NDcsImhlaWdodCI6MS4zMjg2Njc0MDIyNjc0NTZ9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6NDAuODczNzU2NDA4NjkxNDA2LCJsZWZ0IjoxMC41MDE3MjcxMDQxODcwMTIsIndpZHRoIjozOS43MjExNTMyNTkyNzczNDQsImhlaWdodCI6MS4zMjg2Njc0MDIyNjc0NTZ9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6NDIuNjYzNjA4NTUxMDI1MzksImxlZnQiOjEwLjUwMTcyNzEwNDE4NzAxMiwid2lkdGgiOjM5LjcwMzEzNjQ0NDA5MTgsImhlaWdodCI6MS4zMjg2Njc0MDIyNjc0NTZ9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6NDQuNDQ2MjA4OTUzODU3NDIsImxlZnQiOjEwLjUwMTcyNzEwNDE4NzAxMiwid2lkdGgiOjM5LjcwMDc3NTE0NjQ4NDM3NSwiaGVpZ2h0IjoxLjMyODY2NzQwMjI2NzQ1Nn19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjo0Ni4yMzYwNjEwOTYxOTE0MDYsImxlZnQiOjEwLjUwMTcyNzEwNDE4NzAxMiwid2lkdGgiOjM5LjcwNzcyNTUyNDkwMjM0NCwiaGVpZ2h0IjoxLjMyODY2NzQwMjI2NzQ1Nn19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjo0OC4wMjU5MTMyMzg1MjUzOSwibGVmdCI6MTAuNTAxNzI3MTA0MTg3MDEyLCJ3aWR0aCI6MjUuOTQxNjY3NTU2NzYyNjk1LCJoZWlnaHQiOjEuMzI4NjY3NDAyMjY3NDU2fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjQ5LjgxNTc2MTU2NjE2MjExLCJsZWZ0IjoxMy4xMjUwMTYyMTI0NjMzNzksIndpZHRoIjozNy4wODExODA1NzI1MDk3NjYsImhlaWdodCI6MS4zMjg2Njc0MDIyNjc0NTZ9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6NTEuNjA1NjEzNzA4NDk2MDk0LCJsZWZ0IjoxMC41MDE3MjcxMDQxODcwMTIsIndpZHRoIjozOS43MDkyMDU2Mjc0NDE0MDYsImhlaWdodCI6MS4zMjg2Njc0MDIyNjc0NTZ9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6NTMuMzk1NDYyMDM2MTMyODEsImxlZnQiOjEwLjUwMTcyNzEwNDE4NzAxMiwid2lkdGgiOjM5LjcxOTIyMzAyMjQ2MDk0LCJoZWlnaHQiOjEuMzI4NjY3NDAyMjY3NDU2fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjU1LjE4NTMxMDM2Mzc2OTUzLCJsZWZ0IjoxMC41MDE3MjcxMDQxODcwMTIsIndpZHRoIjozOS43MTA2NzgxMDA1ODU5NCwiaGVpZ2h0IjoxLjMyODY2NzQwMjI2NzQ1Nn19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjE2In19XQ%3D%3D. Governance lapses, including underfunded STPs (only 30% operational in peri-urban areas) and weak non-point source controls, allow untreated sewage rich in surfactants and organics to fuel foam plumes and hypoxia (Gobler et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkNocmlzdG9waGVyIEouIiwiZmFtaWx5IjoiR29ibGVyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlJ1dGggVy4iLCJmYW1pbHkiOiJEcmlua3dhdGVyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkEuIFcuIiwiZmFtaWx5IjoiQW50aG9ueSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJKZW5uaWZlciBBLiIsImZhbWlseSI6IkdvbGVza2kifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW5uIE1hcmllIEUuIiwiZmFtaWx5IjoiRmFtdWxhcm8tUGVjb3JhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1pY2hhZWwgQi4iLCJmYW1pbHkiOiJXYWxsYWNlIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5vcmEgUi4gVy4iLCJmYW1pbHkiOiJTdHJhcXVhZGluZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJSb25vam95IiwiZmFtaWx5IjoiSGVtIn19XSwiaWRzIjp7Imlzc24iOlsiMTY2NC0zMDJYIl0sImlzYm4iOltdLCJkb2kiOiIxMC4zMzg5L2ZtaWNiLjIwMjQuMTQ2NDY4NiIsInBtaWQiOiIzOTU2ODk5NyIsIm9wZW5hbGV4IjoiVzQ0MDQxMjA5NTIiLCJleGEiOiJodHRwczovL3d3dy5mcm9udGllcnNpbi5vcmcvam91cm5hbHMvbWljcm9iaW9sb2d5L2FydGljbGVzLzEwLjMzODkvZm1pY2IuMjAyNC4xNDY0Njg2L2Z1bGwifSwidGl0bGUiOiJTZXdhZ2UtYW5kIGZlcnRpbGl6ZXItZGVyaXZlZCBudXRyaWVudHMgYWx0ZXIgdGhlIGludGVuc2l0eSwgZGl2ZXJzaXR5LCBhbmQgdG94aWNpdHkgb2YgaGFybWZ1bCBjeWFub2JhY3RlcmlhbCBibG9vbXMgaW4gZXV0cm9waGljIGxha2VzIiwiYWJzdHJhY3QiOiJDeWFub2JhY3RlcmlhbCBoYXJtZnVsIGFsZ2FsIGJsb29tcyAoQ0hBQnMpIGFyZSBwcm9tb3RlZCBieSBleGNlc3NpdmUgbnV0cmllbnQgbG9hZGluZyBhbmQsIHdoaWxlIGZlcnRpbGl6ZXJzIGFuZCBzZXdhZ2UgYXJlIHRoZSBtb3N0IHByZXZhbGVudCBleHRlcm5hbCBudXRyaWVudCBzb3VyY2VzIGluIG1vc3Qgd2F0ZXJzaGVkcywgdGhlIGRpZmZlcmVudGlhbCBlZmZlY3RzIG9mIHRoZXNlIG51dHJpZW50IHNvdXJjZXMgb24gQ0hBQnMgYXJlIHVua25vd24uIEhlcmUsIHdlIHRyYWNrZWQgQ0hBQnMgYW5kIHBlcmZvcm1lZCBleHBlcmltZW50cyBpbiBmaXZlIGRpc3RpbmN0IGxha2VzIGFjcm9zcyB0aGUgTm9ydGhlcm4gVVMgaW5jbHVkaW5nIExha2UgRXJpZS4gRmVydGlsaXplcnMgd2l0aCBhbW1vbml1bSBhbmQgb3J0aG9waG9zcGhhdGUsIG1lbWJyYW5lICgwLjIgzrxtKS1maWx0ZXJlZCBzZXdhZ2UgKGRvbWluYXRlZCBieSByZWR1Y2VkIGZvcm1zIG9mIG5pdHJvZ2VuKSBzYW5kLWFuZCBtZW1icmFuZS1maWx0ZXJlZCBzZXdhZ2UgKGRvbWluYXRlZCBieSBuaXRyYXRlKSwgYW5kIGFuIGlub3JnYW5pYyBudXRyaWVudCBzb2x1dGlvbiBvZiBhbW1vbml1bSBhbmQgb3J0aG9waG9zcGhhdGUgd2VyZSB1c2VkIGFzIGV4cGVyaW1lbnRhbCBudXRyaWVudCBzb3VyY2VzIGZvciBDSEFCcyBhdCBOLWVxdWl2YWxlbnQsIGVudmlyb25tZW50YWxseSByZWFsaXN0aWMgY29uY2VudHJhdGlvbnMuIFBoeXRvcGxhbmt0b24gY29tbXVuaXRpZXMgd2VyZSBldmFsdWF0ZWQgZmx1b3JvbWV0cmljYWxseSwgbWljcm9zY29waWNhbGx5LCBhbmQgdmlhIGhpZ2ggdGhyb3VnaHB1dCBzZXF1ZW5jaW5nIG9mIHRoZSAxNlMgclJOQSBnZW5lLCBhbmQgbGV2ZWxzIG9mIG1pY3JvY3lzdGluIGFuZCB0aGUgzrQgMTUgTiBjb250ZW50IG9mIHBhcnRpY3VsYXRlIG9yZ2FuaWMgbml0cm9nZW4gKM60UE8gMTUgTikgd2VyZSBxdWFudGlmaWVkLiBGZXJ0aWxpemVyIGFuZCBib3RoIHNvdXJjZXMgb2Ygd2FzdGV3YXRlciBpbmNyZWFzZWQgdGhlIGFidW5kYW5jZSBvZiBjeWFub2JhY3RlcmlhIGluIGFsbCBleHBlcmltZW50cyBhY3Jvc3MgYWxsIGZpdmUgbGFrZXMgKCBwICZhbXA7bHQ7IDAuMDUgZm9yIGFsbCkgd2hlcmVhcyBlZmZlY3RzIG9uIGV1a2FyeW90aWMgcGh5dG9wbGFua3RvbiB3ZXJlIGxpbWl0ZWQuIFNhbmQtZmlsdGVyZWQgc2V3YWdlIGNvbnRhaW5lZCBsZXNzIFAsIG9yZ2FuaWMgbWF0dGVyLCBhbmQgYW1tb25pdW0gYnV0IG1vcmUgbml0cmF0ZSBhbmQgaGFkIGEgMjUlIGxlc3MgcG90ZW50IHN0aW11bGF0b3J5IGVmZmVjdCBvbiBjeWFub2JhY3RlcmlhIHRoYW4gbWVtYnJhbmUtZmlsdGVyZWQgc2V3YWdlLCBzdWdnZXN0aW5nIG5pdHJpZmljYXRpb24gbWF5IHBsYXkgYSByb2xlIGluIHJlZHVjaW5nIENIQUJzLiBGZXJ0aWxpemVyIGluY3JlYXNlZCBtaWNyb2N5c3RpbiBsZXZlbHMgYW5kIGRlY3JlYXNlZCB0aGUgzrRQTyAxNSBOIHdoZXJlYXMgd2FzdGV3YXRlciBpbmNyZWFzZWQgzrRQTyAxNSBOICggcCAmYW1wO2x0OyAwLjA1IGZvciBhbGwpLiBNaWNyb2N5c3RpcyB3YXMgdGhlIGdlbnVzIG1vc3QgY29uc2lzdGVudGx5IHByb21vdGVkIGJ5IG51dHJpZW50IHNvdXJjZXMgKCBwICZhbXA7bHQ7IDAuMDUgaW4gYWxsIGV4cGVyaW1lbnRzKSwgZm9sbG93ZWQgYnkgQ3lhbm9iaXVtICggcCAmYW1wO2x0OyAwLjA1IGluIDUwJSBvZiBleHBlcmltZW50cyksIHdpdGggaW5jcmVhc2VzIGluIE1pY3JvY3lzdGlzIGJpb21hc3MgY29uc2lzdGVudGx5IGVsaWNpdGVkIGJ5IG1lbWJyYW5lLWZpbHRlcmVkIHdhc3Rld2F0ZXIuIENvbGxlY3RpdmVseSwgcmVzdWx0cyBkZW1vbnN0cmF0ZSB0aGF0IGRpZmZlcmluZyB0eXBlcyBvZiBzZXdhZ2UgZGlzY2hhcmdlIGFuZCBmZXJ0aWxpemVycyBjYW4gcHJvbW90ZSBDSEFCIGludGVuc2l0eSBhbmQgdG94aWNpdHksIHdoaWxlIGNvbmN1cnJlbnRseSBhbHRlcmluZyBDSEFCIGRpdmVyc2l0eSBhbmQgzrRQTyAxNSBOLiBXaGlsZSBtZW1icmFuZS1maWx0ZXJlZCBzZXdhZ2UgY29uc2lzdGVudGx5IGZhdm9yZWQgTWljcm9jeXN0aXMgLCB0aGUgZGlzY2hhcmdlIG9mIHNld2FnZSB0aHJvdWdoIHNhbmRzIG11dGVkIGJsb29tIGludGVuc2l0eSBzdWdnZXN0aW5nIHNhbmQtYmVkcyBtYXkgcmVwcmVzZW50IGEgdG9vbCB0byByZW1vdmUga2V5IG51dHJpZW50cyBhbmQgcGFydGlhbGx5IG1pdGlnYXRlIENIQUJzLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzg5L2ZtaWNiLjIwMjQuMTQ2NDY4NiIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cuZnJvbnRpZXJzaW4ub3JnL2pvdXJuYWxzL21pY3JvYmlvbG9neS9hcnRpY2xlcy8xMC4zMzg5L2ZtaWNiLjIwMjQuMTQ2NDY4Ni9wZGYiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMS0wNiIsImFjY2Vzc2VkRGF0ZSI6IjIwMjYtMDIiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZyb250aWVycyBpbiBNaWNyb2Jpb2xvZ3kiLCJ2b2x1bWUiOjE1LCJpbXBhY3RGYWN0b3IiOiIzLjk5In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTQ2NDY4NiwibGFzdFBhZ2UiOjE0NjQ2ODZ9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRnJvbnRpZXJzIE1lZGlhIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo3MywiY2l0ZWRCeUNvdW50Ijo5fX0sInF1b3RlIjoiZGVyaXZlZCBudXRyaWVudHMgYWx0ZXIgdGhlIGludGVuc2l0eSwgZGl2ZXJzaXR5LCBhbmQgdG94aWNpdHkgb2YgaGFybWZ1bCBjeWFub2JhY3RlcmlhbCBibG9vbXMgLi4uIEN5YW5vYmFjdGVyaWFsIGhhcm1mdWwgYWxnYWwgYmxvb21zIChDSEFCcykgYXJlIHByb21vdGVkIGJ5IGV4Y2Vzc2l2ZSBudXRyaWVudCBsb2FkaW5nIGFuZCwgd2hpbGUgZmVydGlsaXplcnMgYW5kIHNld2FnZSBhcmUgdGhlIG1vc3QgcHJldmFsZW50IGV4dGVybmFsIG51dHJpZW50IHNvdXJjZXMgaW4gbW9zdCB3YXRlcnNoZWRzLCB0aGUgZGlmZmVyZW50aWFsIGVmZmVjdHMgb2YgdGhlc2UgbnV0cmllbnQgLi4uIG9yZ2FuaWMgbml0cm9nZW4gKM60UE8xNU4pIHdlcmUgcXVhbnRpZmllZC4gRmVydGlsaXplciBhbmQgYm90aCBzb3VyY2VzIG9mIHdhc3Rld2F0ZXIgaW5jcmVhc2VkIHRoZSBhYnVuZGFuY2Ugb2YgLi4uIHBoeXRvcGxhbmt0b24gd2VyZSBsaW1pdGVkLiBTYW5kLWZpbHRlcmVkIHNld2FnZSBjb250YWluZWQgbGVzcyBQLCBvcmdhbmljIG1hdHRlciwgYW5kIGFtbW9uaXVtIGJ1dCBtb3JlIG5pdHJhdGUgYW5kIC4uLiBkaWZmZXJpbmcgdHlwZXMgb2Ygc2V3YWdlIGRpc2NoYXJnZSBhbmQgZmVydGlsaXplcnMgY2FuIHByb21vdGUgQ0hBQiBpbnRlbnNpdHkgYW5kIHRveGljaXR5LCB3aGlsZSBjb25jdXJyZW50bHkgLi4uIHNld2FnZSB0aHJvdWdoIHNhbmRzIG11dGVkIGJsb29tIGludGVuc2l0eSBzdWdnZXN0aW5nIHNhbmQtYmVkcyBtYXkgcmVwcmVzZW50IGEgdG9vbCB0byByZW1vdmUga2V5IG51dHJpZW50cyBhbmQgLi4uICggV2F0c29uIGV0IGFsLiwgMjAxNiA7IEJ1bGxlcmphaG4gZXQgYWwuLCAyMDE2IDsgSHVpc21hbiBldCBhbC4sIDIwMTggKSBhbmQgZXhjZXNzaXZlIG51dHJpZW50IGxvYWRpbmcgaXMgdGhlIHByb2Nlc3MgbW9zdCBjb21tb25seSBjaXRlZCBhcyB0aGUgZW52aXJvbm1lbnRhbCBmYWN0b3IgcHJvbW90aW5nIENIQUJzICgifV0%3D. Climate feedbacks exacerbate this: warming prolongs stratification, enhancing internal nutrient loading and toxin production, outpacing static BMPs (Meerhoff et al., 2022; Paerl et al., 2019)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhhbnMgVy4iLCJmYW1pbHkiOiJQYWVybCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLYXJsIEUuIiwiZmFtaWx5IjoiSGF2ZW5zIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hdGhhbiBTLiIsImZhbWlseSI6IkhhbGwifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiVGltb3RoeSBHLiIsImZhbWlseSI6Ik90dGVuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1lbmd5dWFuIiwiZmFtaWx5IjoiWmh1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhhaSIsImZhbWlseSI6Ilh1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikd1YW5nd2VpIiwiZmFtaWx5IjoiWmh1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkJvcWlhbmciLCJmYW1pbHkiOiJRaW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIxMzIzLTE2NTAiLCIxNDQ4LTYwNTkiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwNzEvbWYxODM5MiIsIm1hZyI6IjI5NDIzMjE1NDIiLCJvcGVuYWxleCI6IlcyOTQyMzIxNTQyIn0sInRpdGxlIjoiTWl0aWdhdGluZyBhIGdsb2JhbCBleHBhbnNpb24gb2YgdG94aWMgY3lhbm9iYWN0ZXJpYWwgYmxvb21zOiBjb25mb3VuZGluZyBlZmZlY3RzIGFuZCBjaGFsbGVuZ2VzIHBvc2VkIGJ5IGNsaW1hdGUgY2hhbmdlIiwiYWJzdHJhY3QiOiJNYW5hZ2luZyBhbmQgbWl0aWdhdGluZyB0aGUgZ2xvYmFsIGV4cGFuc2lvbiBvZiB0b3hpYyBjeWFub2JhY3RlcmlhbCBoYXJtZnVsIGFsZ2FsIGJsb29tcyAoQ3lhbm9IQUJzKSBpcyBhIG1ham9yIGNoYWxsZW5nZSBmYWNpbmcgcmVzZWFyY2hlcnMgYW5kIHdhdGVyIHJlc291cmNlIG1hbmFnZXJzLiBWYXJpb3VzIGFwcHJvYWNoZXMsIGluY2x1ZGluZyBudXRyaWVudCBsb2FkIHJlZHVjdGlvbiwgYXJ0aWZpY2lhbCBtaXhpbmcgYW5kIGZsdXNoaW5nLCBvbW5pdm9yb3VzIGZpc2ggcmVtb3ZhbCwgYWxnYWVjaWRlIGFwcGxpY2F0aW9ucyBhbmQgc2VkaW1lbnQgZHJlZGdpbmcsIGhhdmUgYmVlbiB1c2VkIHRvIHJlZHVjZSBibG9vbSBvY2N1cnJlbmNlcy4gSG93ZXZlciwgbWFuYWdlcnMgbm93IGZhY2UgdGhlIGFkZGl0aW9uYWwgY2hhbGxlbmdlIG9mIGhhdmluZyB0byBhZGRyZXNzIHRoZSBlZmZlY3RzIG9mIGNsaW1hdGUgY2hhbmdlIG9uIHdhdGVyc2hlZCBoeWRyb2xvZ2ljYWwgYW5kIG51dHJpZW50IGxvYWQgZHluYW1pY3MsIHdhdGVyIHRlbXBlcmF0dXJlLCBtaXhpbmcgcmVnaW1lIGFuZCBpbnRlcm5hbCBudXRyaWVudCBjeWNsaW5nLiBSaXNpbmcgdGVtcGVyYXR1cmVzIGFuZCBpbmNyZWFzaW5nIGZyZXF1ZW5jaWVzIGFuZCBtYWduaXR1ZGVzIG9mIGV4dHJlbWUgd2VhdGhlciBldmVudHMsIGluY2x1ZGluZyB0cm9waWNhbCBjeWNsb25lcywgZXh0cmF0cm9waWNhbCBzdG9ybXMsIGZsb29kcyBhbmQgZHJvdWdodHMsIGFsbCBwcm9tb3RlIEN5YW5vSEFCcyBhbmQgYWZmZWN0IHRoZSBlZmZpY2FjeSBvZiBlY29zeXN0ZW0gcmVtZWRpYXRpb24gbWVhc3VyZXMuIFRoZXNlIGNsaW1hdGljIGNoYW5nZXMgd2lsbCBsaWtlbHkgcmVxdWlyZSBzZXR0aW5nIHN0cmljdGVyIG51dHJpZW50IChpbmNsdWRpbmcgYm90aCBuaXRyb2dlbiBhbmQgcGhvc3Bob3J1cykgcmVkdWN0aW9uIHRhcmdldHMgZm9yIGJsb29tIGNvbnRyb2wgaW4gYWZmZWN0ZWQgd2F0ZXJzLiBJbiBhZGRpdGlvbiwgdGhlIGVmZmljYWN5IG9mIGN1cnJlbnRseSB1c2VkIG1ldGhvZHMgdG8gcmVkdWNlIEN5YW5vSEFCcyB3aWxsIG5lZWQgdG8gYmUgcmUtZXZhbHVhdGVkIGluIGxpZ2h0IG9mIHRoZSBzeW5lcmdpc3RpYyBlZmZlY3RzIG9mIGNsaW1hdGUgY2hhbmdlIHdpdGggbnV0cmllbnQgZW5yaWNobWVudC4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTA3MS9tZjE4MzkyIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2Nkci5saWIudW5jLmVkdS9kb3dubG9hZHMvMDU3NDIyODM0IiwiaXNzdWVkRGF0ZSI6IjIwMTktMDMtMjUiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTEwIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJNYXJpbmUgYW5kIEZyZXNod2F0ZXIgUmVzZWFyY2giLCJpc3N1ZSI6NSwidm9sdW1lIjo3MSwiaW1wYWN0RmFjdG9yIjoiMi43MCJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjU3OSwibGFzdFBhZ2UiOjU3OX19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJDU0lSTyBQdWJsaXNoaW5nIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjoxMDQsImNpdGVkQnlDb3VudCI6OTF9fSwicXVvdGUiOiJUaGF0IHJlbGF0aW9uc2hpcCBoYXMgc2VydmVkIGFzIHRoZSBmb3VuZGF0aW9uIGZvciBzZXR0aW5nICBudXRyaWVudCBsb2FkaW5nIHRhcmdldHMgaW4gbGFrZXMgYXJvdW5kIHRoZSB3b3JsZCwgYW5kIGNhcmVmdWxseSAgZGVzaWduZWQgZXhwZXJpbWVudHMgbWF5IGVsdWNpZGF0ZSB0byB3aGF0IGV4dGVudCB0ZW1wZXJhdHVyZSAgaW5jcmVhc2UgYWZmZWN0cyB0aGUgdGhyZXNob2xkIGF0IHdoaWNoIHRoZSBmcmVxdWVuY3kgb2Ygb2NjdXIgIHJlbmNlIG9mIEN5YW5vSEFCcyBxdWlja2x5IHJpc2VzIHdpdGggaW5jcmVhc2luZyBOIGFuZCBQLiBDb25jbHVzaW9ucyBSZWNlbnQgc3R1ZGllcyBpbmRpY2F0ZSB0aGF0IHRoZSBzeW5lcmdpc3RpYyBjb21iaW5hdGlvbiBvZiBhbnRocm9wb2dlbmljIG51dHJpZW50IGxvYWRpbmcsIHJpc2luZyB0ZW1wZXJhdHVyZXMsIGVuaGFuY2VkICB2ZXJ0aWNhbCBzdHJhdGlmaWNhdGlvbiwgaW5jcmVhc2VkIHJlc2lkZW5jZSB0aW1lIGFuZCBtb3JlICBleHRyZW1lIGNsaW1hdGljIGNvbmRpdGlvbnMgb3ZlcmFsbCB3aWxsIGZhdm91ciBjeWFub2JhY3RlcmlhbCAgZG9taW5hbmNlIGluIGEgd2lkZSByYW5nZSBvZiBhcXVhdGljIGVjb3N5c3RlbXMuIFRoZSBleHBhbiAgc2lvbiBvZiBDeWFub0hBQnMgaGFzIHNlcmlvdXMgY29uc2VxdWVuY2VzIGZvciBodW1hbiB3YXRlciAgc3VwcGxpZXMsIGZpc2hlcmllcyBhbmQgcmVjcmVhdGlvbmFsIHJlc291cmNlcy4gRWNvc3lzdGVtIHNjYWxlICBwaHlzaWNhbCBjaGVtaWNhbCBiaW90aWMgbWl0aWdhdGlvbiBzdHJhdGVnaWVzIHdpbGwgaGF2ZSB0byAgaW5jb3Jwb3JhdGUgbnV0cmllbnQgKG1vc3Qgb2Z0ZW4gYm90aCBOIGFuZCBQKSBpbnB1dCByZWR1Y3Rpb25zLiAgVGhlIG1hZ25pdHVkZXMgb2YgdGhlc2UgcmVkdWN0aW9ucyB3aWxsIG5lZWQgdG8gYmUgc3lzdGVtICBzcGVjaWZpYyBhbmQgd2lsbCBsaWtlbHkgbmVlZCB0byBiZSBhZGp1c3RlZCB3aXRoIGNoYW5naW5nIGNsaSAgbWF0aWMgY29uZGl0aW9ucy4gU3BlY2lmaWNhbGx5LCB3YXRlciBxdWFsaXR5IG1hbmFnZXJzIHdpbGwgaGF2ZSAgdG8gYWNjb21tb2RhdGUgd2FybWluZyBhbmQgZ3JlYXRlciBoeWRyb2xvZ2ljYWwgdmFyaWFiaWxpdHkgIGFuZCBleHRyZW1lbmVzcyByZXN1bHRpbmcgZnJvbSBjbGltYXRlIGNoYW5nZSBpbiBmb3JtdWxhdGluZyAgdGhlaXIgc3RyYXRlZ2llcy4gQSBrZXkgbG9uZyB0ZXJtIGNvbnRyb2wgd2UgY2FuIGV4ZXJ0IHRvIHJlZHVjZSAgdGhlIHJhdGUgYW5kIGV4dGVudCBvZiBnbG9iYWwgd2FybWluZyBpcyBjdXJiaW5nIGdyZWVuaG91c2UgZ2FzICBlbWlzc2lvbnMuIFdpdGhvdXQgdGhpcyBlc3NlbnRpYWwgc3RlcCwgZnV0dXJlIHdhcm1pbmcgdHJlbmRzLCAgaHlkcm9sb2dpY2FsIGV4dHJlbWVzIChyZWNvcmQgcmFpbmZhbGxzIGFuZCBkcm91Z2h0cykgYW5kIHRoZWlyICBlZmZlY3RzIG9uIGFxdWF0aWMgZWNvc3lzdGVtcyB3aWxsIGNvbnRpbnVlIHRvIHByb21vdGUgb3Bwb3IgIHR1bmlzdGljIEN5YW5vSEFCcy4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6MTQuMTQ5MjMzODE4MDU0MiwibGVmdCI6MTMuMTI4NTk1MzUyMTcyODUyLCJ3aWR0aCI6MzcuMDgyMTcyMzkzNzk4ODMsImhlaWdodCI6MS41MTY2MDIyNzc3NTU3MzczfX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjE1LjQ1MTUwODUyMjAzMzY5MSwibGVmdCI6MTMuMTI4NTQ0ODA3NDM0MDgyLCJ3aWR0aCI6MzcuMDg0ODMxMjM3NzkyOTcsImhlaWdodCI6MS41MTY2MDMyMzE0MzAwNTM3fX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjE2Ljc1Mzc4NDE3OTY4NzUsImxlZnQiOjEzLjEyODU0MjkwMDA4NTQ1LCJ3aWR0aCI6MzcuMDgzMjA5OTkxNDU1MDgsImhlaWdodCI6MS41MTY2MDQxODUxMDQzNzAxfX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjE4LjA1NjA1MzE2MTYyMTA5NCwibGVmdCI6MTMuMTI4NTQ0ODA3NDM0MDgyLCJ3aWR0aCI6MzYuNjE1MDc0MTU3NzE0ODQ0LCJoZWlnaHQiOjEuNTE2NjA0MTg1MTA0MzcwMX19LHsicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjoxOS40NjgzNzgwNjcwMTY2LCJsZWZ0IjoxMy4xMjg1NDQ4MDc0MzQwODIsIndpZHRoIjozNC4yOTA1MzExNTg0NDcyNjYsImhlaWdodCI6MS4yNDUwMTEyMTA0NDE1ODk0fX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjIyLjIyNjE0Mjg4MzMwMDc4LCJsZWZ0IjoxMy4xMjg1NTA1Mjk0Nzk5OCwid2lkdGgiOjcuNjQ5ODczNzMzNTIwNTA4LCJoZWlnaHQiOjEuMTM1NDgzMTQ1NzEzODA2Mn19LHsicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjoyMy43MzQ4NDk5Mjk4MDk1NywibGVmdCI6MTMuMTI4NTM1MjcwNjkwOTE4LCJ3aWR0aCI6MzcuMDg5Mjg2ODA0MTk5MjIsImhlaWdodCI6MjYuMDg4NjQwMjEzMDEyNjk1fX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNTg5In19LHsib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik0uIiwiZmFtaWx5IjoiTWVlcmhvZmYifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSi4iLCJmYW1pbHkiOiJBdWRldCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJULkEuIiwiZmFtaWx5IjoiRGF2aWRzb24ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTC4gRGUiLCJmYW1pbHkiOiJNZWVzdGVyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlMuIiwiZmFtaWx5IjoiSGlsdCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTLiIsImZhbWlseSI6Iktvc3RlbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJaYWlodWEiLCJmYW1pbHkiOiJMaXUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTi4iLCJmYW1pbHkiOiJNYXp6ZW8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSGFucyBXLiIsImZhbWlseSI6IlBhZXJsIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik0uIiwiZmFtaWx5IjoiU2NoZWZmZXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiRS4iLCJmYW1pbHkiOiJKZXBwZXNlbiJ9fV0sImlkcyI6eyJpc3NuIjpbXSwiaXNibiI6W10sImRvaSI6IjEwLjE3NjE1Lzd2ZTUtdzAxNiIsIm9wZW5hbGV4IjoiVzQzMTg0MTU1NzcifSwidGl0bGUiOiJGZWVkYmFjayBiZXR3ZWVuIGNsaW1hdGUgY2hhbmdlIGFuZCBldXRyb3BoaWNhdGlvbjogcmV2aXNpdGluZyB0aGUgYWxsaWVkIGF0dGFjayBjb25jZXB0IGFuZCBob3cgdG8gc3RyaWtlIGJhY2siLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTc2MTUvN3ZlNS13MDE2IiwicmVzb3VyY2VVcmwiOiJodHRwczovL2Nkci5saWIudW5jLmVkdS9kb3dubG9hZHMvaDk4OXJjOTM4IiwiaXNzdWVkRGF0ZSI6IjIwMjItMDEtMDEiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTEwIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJDYXJvbGluYSBEaWdpdGFsIFJlcG9zaXRvcnkgKFVuaXZlcnNpdHkgb2YgTm9ydGggQ2Fyb2xpbmEgYXQgQ2hhcGVsIEhpbGwpIiwiaW1wYWN0RmFjdG9yIjoiMC4xMCJ9LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJVbml2ZXJzaXR5IG9mIE5vcnRoIENhcm9saW5hIGF0IENoYXBlbCBIaWxsIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjowLCJjaXRlZEJ5Q291bnQiOjN9fSwicXVvdGUiOiJXaXRoIGNsaW1hdGUgd2FybWluZywgdGhlIGRpcmVjdCBhbmQgaW5kaS1yZWN0IGVmZmVjdHMgb2YgaW5jcmVhc2VkIGxha2UgdGVtcGVyYXR1cmVzIHdpbGwgbGlrZWx5IGRyaXZlIGNoYW5nZXMgaW4gY3lhbm9iYWN0ZXJpYWwgdGF4YSBhbmQgdGhlaXIgcHJvZHVjLXRpb24gb2YgY3lhbm9iYWN0ZXJpYWwgdG94aW5zLCBsaWtlbHkgaW5jbHVkaW5nIHByb21vdC1pbmcgdGhlIHNlbGVjdGlvbiBvZiBhIGZldyBNaWNyb2N5c3RpcyBzcGVjaWVzIG9yIHN0cmFpbnMgcHJvZHVjaW5nIGhpZ2hseSB0b3hpYyBtaWNyb2N5c3Rpbi1MUiAoTWFudHpvdWtpIGV0IGFsLiAyMDE4KS4gRGVzcGl0ZSB2YXJpYXRpb25zIGFtb25nIGxha2VzIGluIGEgd2F0ZXItc2NhcGUsIGEgcmVnaW9uYWwgaW5jcmVhc2UgaW4gbWljcm9jeXN0aW4gbWF4aW1hIGFuZCBkdXJhdGlvbiBvZiBibG9vbXMgaGFzIGFsc28gYmVlbiBjbGVhcmx5IGV4cGxhaW5lZCBieSBpbmNyZWFzaW5nIHRlbXBlcmF0dXJlIGluIGNlbnRyYWwgTm9ydGggQW1lcmljYSAoSGF5ZXMgZXQgYWwuIDIwMjApLCB3aXRoIGluY3JlYXNpbmcgcmlza3MgdG8gaHVtYW4gaGVhbHRoLiBHcm93aW5nIGV2aWRlbmNlIGluZGljYXRlcyB0aGF0IGN5YW5vYmFjdGVyaWFsIGRvbWluYW5jZSB3aWxsIGV4ZXJ0IGEgcG9zaXRpdmUgZmVlZGJhY2sgd2l0aCBjbGltYXRlIHdhcm1pbmcgKEhhdmVucyBldCBhbC4gMjAxNiwgUGFlcmwgZXQgYWwuIDIwMjApLiBNYW55IGN5YW5vYmFjdGVyaWFsIHNwZWNpZXMgcHJvbGlmZXJhdGUgb24gdGhlIGxha2Ugc3VyZmFjZSwgYWJzb3JiaW5nIGxpZ2h0LCBlbmhhbmNpbmcgbG9jYWxpemVkIHRlbXBlcmF0dXJlIGluY3JlYXNlcywgYW5kIGluY3JlYXNpbmcgdGhlcm1hbCBzdHJhdGlmaWNhdGlvbiBwb3RlbnRpYWwgKEthaHJ1IGV0IGFsLiAxOTkzKS4gQ3lhbm9iYWN0ZXJpYSBwcm9tb3RlIHRoZSByZWxlYXNlIG9mIEdIR3MsIHBhcnRpY3VsYXJseSBDSDQgKFlhbiBldCBhbC4gMjAxNyksIGR1cmluZyB0aGUgZGVjb21wb3NpdGlvbiBmb2xsb3dpbmcgdGhlIGNvbGxhcHNlIG9mIGEgYmxvb20uIEN5YW5vYmFjdGVyaWEgYWxzbyBwcm9tb3RlIHJlbWluZXJhbGl6YXRpb24gb2YgbnV0cmktZW50cyBkdXJpbmcgYmxvb21zLCBsZWFkaW5nIHRvIG11bHRpcGxlIGJsb29tcyBpbiBhIHNpbmdsZSBzZWFzb24uIFVuZGVyIGV1dHJvcGhpYyBhbmQgd2FybWluZyBjb25kaS10aW9ucywgY3lhbm9iYWN0ZXJpYSBkb21pbmFuY2UgbWF5IHRodXMgYmUgYSBzZWxmLXBlcnBldHVhdGluZyBwcm9waGVjeSAoRmlnLiAyIGFuZCAzKS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo2Ny4wNjM0MDAyNjg1NTQ2OSwibGVmdCI6NTEuNDc4ODAxNzI3Mjk0OTIsIndpZHRoIjozOC42NzM0MjM3NjcwODk4NDQsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6NjguNjk4NTQ3MzYzMjgxMjUsImxlZnQiOjUxLjQ3ODgwMTcyNzI5NDkyLCJ3aWR0aCI6MzguNjc5NDk2NzY1MTM2NzIsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6NzAuMzMzNjg2ODI4NjEzMjgsImxlZnQiOjUxLjQ3ODgwMTcyNzI5NDkyLCJ3aWR0aCI6MzguNjc3NTgxNzg3MTA5Mzc1LCJoZWlnaHQiOjEuNDcxMTAxNTIyNDQ1Njc4N319LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjcxLjk2ODgyNjI5Mzk0NTMxLCJsZWZ0Ijo1MS40Nzg4MDE3MjcyOTQ5Miwid2lkdGgiOjM4LjY3OTEwMzg1MTMxODM2LCJoZWlnaHQiOjEuNDcxMTAxNTIyNDQ1Njc4N319LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjczLjYxMDU3MjgxNDk0MTQsImxlZnQiOjUxLjQ3ODgwMTcyNzI5NDkyLCJ3aWR0aCI6MzguNjc0NDI3MDMyNDcwNywiaGVpZ2h0IjoxLjQ3Mzc0Njg5NTc5MDF9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo3NS4yNDcwMzk3OTQ5MjE4OCwibGVmdCI6NTEuNDc4ODA5MzU2Njg5NDUsIndpZHRoIjozOC42NzQyNjMwMDA0ODgyOCwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo3Ni44ODIxODY4ODk2NDg0NCwibGVmdCI6NTEuNDc4ODA5MzU2Njg5NDUsIndpZHRoIjozOC42ODExNzE0MTcyMzYzMywiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo3OC41MjUyNTMyOTU4OTg0NCwibGVmdCI6NTEuNDc4ODA5MzU2Njg5NDUsIndpZHRoIjozOC42NzU2NjI5OTQzODQ3NjYsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODAuMTYwNDAwMzkwNjI1LCJsZWZ0Ijo1MS40Nzg4MDkzNTY2ODk0NSwid2lkdGgiOjM4LjY4MjAyOTcyNDEyMTA5NCwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo4MS43OTU1Mzk4NTU5NTcwMywibGVmdCI6NTEuNDc4ODA5MzU2Njg5NDUsIndpZHRoIjozOC42NzQ5ODM5NzgyNzE0ODQsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODMuNDM4NjEzODkxNjAxNTYsImxlZnQiOjUxLjQ3ODgwOTM1NjY4OTQ1LCJ3aWR0aCI6MzguNjc2MTg1NjA3OTEwMTU2LCJoZWlnaHQiOjEuNDg2OTczNTI0MDkzNjI4fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODUuMDg5NjMwMTI2OTUzMTIsImxlZnQiOjUxLjQ3ODgwOTM1NjY4OTQ1LCJ3aWR0aCI6NC42ODI4MDUwNjEzNDAzMzIsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODYuNzI0Nzc3MjIxNjc5NjksImxlZnQiOjUzLjQ0MTE3NzM2ODE2NDA2LCJ3aWR0aCI6Ni4xNDAzNTU1ODcwMDU2MTUsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODYuNzI0ODkxNjYyNTk3NjYsImxlZnQiOjYwLjk5NDg3Njg2MTU3MjI2Niwid2lkdGgiOjYuMDM2OTkyNTQ5ODk2MjQsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODYuNzI0ODkxNjYyNTk3NjYsImxlZnQiOjY4LjQ0NjUyNTU3MzczMDQ3LCJ3aWR0aCI6Ni4xNTA3MDU4MTQzNjE1NzIsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODYuNzI0ODkxNjYyNTk3NjYsImxlZnQiOjc2LjAwOTgyNjY2MDE1NjI1LCJ3aWR0aCI6Mi43MjczMzMzMDcyNjYyMzU0LCJoZWlnaHQiOjEuNDcxMTAxNTIyNDQ1Njc4N319LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjg2LjcyNDg5MTY2MjU5NzY2LCJsZWZ0Ijo4MC4xNDk1ODE5MDkxNzk2OSwid2lkdGgiOjEwLjAwODIzNDk3NzcyMjE2OCwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo4OC4zNjAwMzg3NTczMjQyMiwibGVmdCI6NTEuNDc3NjU3MzE4MTE1MjM0LCJ3aWR0aCI6MzguNjcyMDU0MjkwNzcxNDg0LCJoZWlnaHQiOjEuNDcxMTAxNTIyNDQ1Njc4N319LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjkwLjAwMzExMjc5Mjk2ODc1LCJsZWZ0Ijo1MS40Nzc2NTczMTgxMTUyMzQsIndpZHRoIjozOC42NzY0OTg0MTMwODU5NCwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo5MS42MzgyNTIyNTgzMDA3OCwibGVmdCI6NTEuNDc3NjUzNTAzNDE3OTcsIndpZHRoIjozOC42NzU4MzQ2NTU3NjE3MiwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjksImJib3giOnsidG9wIjo2LjQ2ODkzNTk2NjQ5MTY5OSwibGVmdCI6OS44NDAxMzM2NjY5OTIxODgsIndpZHRoIjozOC42ODYzMjUwNzMyNDIxOSwiaGVpZ2h0IjoxNi4yMTIxMjU3NzgxOTgyNDJ9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI5In19XQ%3D%3D. Economic evaluations reveal overlooked costs, like INR 500-1000 crore fishery losses in Bengaluru, unaddressed by rigid policies (Berdalet et al., 2017)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkVsaXNhIiwiZmFtaWx5IjoiQmVyZGFsZXQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUmFwaGFlbCBNLiIsImZhbWlseSI6Ikt1ZGVsYSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJFZCIsImZhbWlseSI6IlVyYmFuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhlbnJpayIsImZhbWlseSI6IkVuZXZvbGRzZW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTmVpbCBTLiIsImZhbWlseSI6IkJhbmFzIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkVpbGVlbiIsImZhbWlseSI6IkJyZXNuYW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTWljaGVsZSBBLiIsImZhbWlseSI6IkJ1cmZvcmQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiS2VpdGgiLCJmYW1pbHkiOiJEYXZpZHNvbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJDaHJpc3RvcGhlciBKLiIsImZhbWlseSI6IkdvYmxlciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJCZW5ndCIsImZhbWlseSI6Ikthcmxzb24ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUG8gVGVlbiIsImZhbWlseSI6IkxpbSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJMaW5jb2xuIiwiZmFtaWx5IjoiTWFjS2VuemllIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1hcmluYSIsImZhbWlseSI6Ik1vbnRyZXNvciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJWZXJhIEwuIiwiZmFtaWx5IjoiVHJhaW5lciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJHaXJlcyIsImZhbWlseSI6IlVzdXAifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiS2Vkb25nIiwiZmFtaWx5IjoiWWluIn19XSwiaWRzIjp7Imlzc24iOlsiMTA0Mi04Mjc1IiwiMjM3Ny02MTdYIl0sImlzYm4iOltdLCJkb2kiOiIxMC41NjcwL29jZWFub2cuMjAxNy4xMTEiLCJtYWciOiIyNTk3NjY0NzI3Iiwib3BlbmFsZXgiOiJXMjU5NzY2NDcyNyJ9LCJ0aXRsZSI6Ikdsb2JhbEhBQjogQSBOZXcgUHJvZ3JhbSB0byBQcm9tb3RlIEludGVybmF0aW9uYWwgUmVzZWFyY2gsIE9ic2VydmF0aW9ucywgYW5kIE1vZGVsaW5nIG9mIEhhcm1mdWwgQWxnYWwgQmxvb21zIGluIEFxdWF0aWMgU3lzdGVtcyIsImFic3RyYWN0IjoiQmVyZGFsZXQsIEVsaXNhIC4uLiBldCBhbC4tLSBTcGVjaWFsIGlzc3VlIG9uIEludGVybmF0aW9uYWwgQ29vcGVyYXRpb24gaW4gSGFybWZ1bCBBbGdhbCBCbG9vbSBTY2llbmNlLi0tIDEyIHBhZ2VzLCA2IGZpZ3VyZXMiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuNTY3MC9vY2Vhbm9nLjIwMTcuMTExIiwicmVzb3VyY2VVcmwiOiJodHRwczovL3Rvcy5vcmcvb2NlYW5vZ3JhcGh5L2Fzc2V0cy9kb2NzLzMwLTFfYmVyZGFsZXQzLnBkZiIsImlzc3VlZERhdGUiOiIyMDE3LTAzLTAxIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiT2NlYW5vZ3JhcGh5IiwiaXNzdWUiOjEsInZvbHVtZSI6MzAsImltcGFjdEZhY3RvciI6IjIuOTMifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjo3MCwibGFzdFBhZ2UiOjgxfX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6Ik9jZWFub2dyYXBoeSBTb2NpZXR5In0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjoyNiwiY2l0ZWRCeUNvdW50IjozMn19LCJxdW90ZSI6IlRoZSAgZWNvbm9taWMgZWZmZWN0cyBvZiBIQUJzIGFyaXNlIGZyb20gcHViLWxpYyBoZWFsdGggY29zdHMsIGNvbW1lcmNpYWwgZmlzaGVyeSBhbmQgIGFxdWFjdWx0dXJlIG9wZXJhdGlvbiBjbG9zdXJlcyBhbmQgZmlzaCAga2lsbHMsIHBvc3NpYmxlIG1lZGl1bS0gYW5kIGxvbmctdGVybSAgZGVjbGluZXMgaW4gY29hc3RhbCBhbmQgbWFyaW5lIHJlY3JlYXRpb24gIGFuZCB0b3VyaXNtLCBhbmQgdGhlIGNvc3RzIG9mIGluc3VyYW5jZSwgIG1vbml0b3JpbmcsIG1hbmFnZW1lbnQsIGFuZCBtaXRpZ2F0aW9uICAoTW9yZ2FuIGV0IGFsLiwgMjAxMCkuIEFnZ3JlZ2F0aW5nIGVjby1ub21pYyBlZmZlY3Rz4oCUYm90aCB3aXRoaW4gYW5kIGFjcm9zcyAgdGhlc2UgY2F0ZWdvcmllc%2BKAlGNhbiBhbHNvIGJlIHByb2JsZW0tYXRpYyB3aXRoIGF2YWlsYWJsZSBkYXRhIGFuZCBjdXJyZW50bHkgdXNlZCAgbWV0aG9kb2xvZ2llcyAoSG9hZ2xhbmQgZXQgYWwuLCAyMDAyKS4gIEFuIGV4YW1wbGUgb2YgYW4gZWNvbm9taWMgZXZhbHVhdGlvbiAgb2YgSEFCcyBhdCBhIG5hdGlvbmFsIGxldmVsIGlzIGdpdmVuIGJ5ICBBbmRlcnNvbiBldCBhbC4gKDIwMDApLCB3aG8gZXN0aW1hdGVkIHRoZSAgZWNvbm9taWMgZWZmZWN0cyBvZiBIQUJzIGluIHRoZSBVbml0ZWQgIFN0YXRlcyB0byBiZSAkMTAwIG1pbGxpb24gcGVyIHllYXIgKGFzIGxhdGVyICB0cmFuc2xhdGVkIGludG8gMjAxMiBkb2xsYXJzKSwgYnJva2VuIGRvd24gIGFzIGZvbGxvd3M6IDQ1JSBmb3IgcHVibGljIGhlYWx0aCBjb3N0cywgIDM3JSBmb3IgdGhlIGNvc3RzIG9mIGNsb3N1cmVzIGFuZCBsb3NzZXMgIGV4cGVyaWVuY2VkIGJ5IGNvbW1lcmNpYWwgZmlzaGVyaWVzLCAgMTMlIGZyb20gdGhlIGltcGFjdCBvbiBsb3N0IHJlY3JlYXRpb24gIGFuZCB0b3VyaXNtIG9wcG9ydHVuaXRpZXMsIGFuZCA0JSBmcm9tIEZJR1VSRSA2LiBNYXNzaXZlIGZpc2ggbW9ydGFsaXRpZXMgaW4gSmFrYXJ0YSwgSW5kb25lc2lhLiBQaG90byBjcmVkaXRzOiBUdW1wYWsgU2lkYWJ1dGFyIChIQU41MykgbW9uaXRvcmluZyBhbmQgbWFuYWdlbWVudCBjb3N0cy4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjoxMS44MTk0NDQ2NTYzNzIwNywibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzQ5NDUyOTcyNDEyMSwiaGVpZ2h0IjoxLjcwMzI4MjU5NDY4MDc4NjF9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjoxMy41MjM5ODk2Nzc0MjkyLCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI2Ljg2NDM3NDE2MDc2NjYsImhlaWdodCI6MS43MDMyODI1OTQ2ODA3ODYxfX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6MTUuMjI4NTM1NjUyMTYwNjQ1LCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzMjAwMjI1ODMwMDc4LCJoZWlnaHQiOjEuNzAzMjgzNTQ4MzU1MTAyNX19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjE2LjkzMzA4MDY3MzIxNzc3MywibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzM2MzY4NTYwNzkxLCJoZWlnaHQiOjEuNzAzMjgzNTQ4MzU1MTAyNX19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjE4LjYzNzYyNjY0Nzk0OTIyLCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzNTI5NDM0MjA0MTAxNiwiaGVpZ2h0IjoxLjcwMzI4MzU0ODM1NTEwMjV9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjoyMC4zNDIxNzA3MTUzMzIwMywibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzM5ODU5MDA4Nzg5MDYsImhlaWdodCI6MS43MDMyODM1NDgzNTUxMDI1fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6MjIuMDQ2NzE2NjkwMDYzNDc3LCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzMjAxMTc5NTA0Mzk0NSwiaGVpZ2h0IjoxLjcwMzI4MzU0ODM1NTEwMjV9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjoyMy43NTEyNjI2NjQ3OTQ5MjIsImxlZnQiOjY3LjI1NDg5ODA3MTI4OTA2LCJ3aWR0aCI6MjcuMjMyMDQyMzEyNjIyMDcsImhlaWdodCI6MS43MDMyODM1NDgzNTUxMDI1fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6MjUuNDU1ODA4NjM5NTI2MzY3LCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI2Ljg2MzA2NzYyNjk1MzEyNSwiaGVpZ2h0IjoxLjcwMzI4MzU0ODM1NTEwMjV9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjoyNy4xNjAzNTI3MDY5MDkxOCwibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzU0MzM1Nzg0OTEyMSwiaGVpZ2h0IjoxLjcwMzI4MzU0ODM1NTEwMjV9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjoyOC44NjQ4OTg2ODE2NDA2MjUsImxlZnQiOjY3LjI1NDg5ODA3MTI4OTA2LCJ3aWR0aCI6MjYuODY0Mzk1MTQxNjAxNTYyLCJoZWlnaHQiOjEuNzAzMjgzNTQ4MzU1MTAyNX19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjMwLjU2OTQ0Mjc0OTAyMzQzOCwibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzQ5MjQzMTY0MDYyNSwiaGVpZ2h0IjoxLjcwMzI4MzU0ODM1NTEwMjV9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjozMi4yNzM5OTA2MzExMDM1MTYsImxlZnQiOjY3LjI1NDg5ODA3MTI4OTA2LCJ3aWR0aCI6MjcuMjM0Nzc1NTQzMjEyODksImhlaWdodCI6MS43MDMyODM1NDgzNTUxMDI1fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6MzMuOTc4NTM0Njk4NDg2MzMsImxlZnQiOjY3LjI1NDg5ODA3MTI4OTA2LCJ3aWR0aCI6MjcuMjM0MzM2ODUzMDI3MzQ0LCJoZWlnaHQiOjEuNzAzMjg1NDU1NzAzNzM1NH19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjM1LjY4MzA3ODc2NTg2OTE0LCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzMjE2MDU2ODIzNzMwNSwiaGVpZ2h0IjoxLjcwMzI4NTQ1NTcwMzczNTR9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjozNy4zODc2MjI4MzMyNTE5NSwibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzM2NjU0NjYzMDg1OTQsImhlaWdodCI6MS43MDMyODU0NTU3MDM3MzU0fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6MzkuMDkyMTcwNzE1MzMyMDMsImxlZnQiOjY3LjI1NDg5ODA3MTI4OTA2LCJ3aWR0aCI6MjcuMjM2NTc5ODk1MDE5NTMsImhlaWdodCI6MS43MDMyODU0NTU3MDM3MzU0fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6NDAuNzk2NzE0NzgyNzE0ODQ0LCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzNjcxNzIyNDEyMTA5NCwiaGVpZ2h0IjoxLjcwMzI4NTQ1NTcwMzczNTR9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo0Mi41MDEyNjI2NjQ3OTQ5MiwibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzM2MjczMTkzMzU5MzgsImhlaWdodCI6MS43MDMyODU0NTU3MDM3MzU0fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6NDQuMjA1ODA2NzMyMTc3NzM0LCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzMDU0NTA0Mzk0NTMxMiwiaGVpZ2h0IjoxLjcwMzI4NTQ1NTcwMzczNTR9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo0NS45MTAzNTA3OTk1NjA1NSwibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzg1NDQ0NjQxMTEzMjgsImhlaWdodCI6MS43MDMyODU0NTU3MDM3MzU0fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6NDcuNjE0ODk0ODY2OTQzMzYsImxlZnQiOjY3LjI1NDg5ODA3MTI4OTA2LCJ3aWR0aCI6MjcuMjM0MTQ2MTE4MTY0MDYyLCJoZWlnaHQiOjEuNzAzMjg1NDU1NzAzNzM1NH19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjQ5LjMxOTQ0Mjc0OTAyMzQ0LCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzNTk0MDkzMzIyNzU0LCJoZWlnaHQiOjEuNzAzMjg1NDU1NzAzNzM1NH19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjUxLjAyMzk5MDYzMTEwMzUxNiwibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzY5MDc5NTg5ODQzNzUsImhlaWdodCI6MS43MDMyODU0NTU3MDM3MzU0fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6OTIuMjc4NzcwNDQ2Nzc3MzQsImxlZnQiOjguODIzNTI5MjQzNDY5MjM4LCJ3aWR0aCI6NTYuMjYzOTE2MDE1NjI1LCJoZWlnaHQiOjEuMjA5MDg5NTE3NTkzMzgzOH19LHsicGFnZU51bWJlciI6OSwiYmJveCI6eyJ0b3AiOjUuMDAxMjYyNjY0Nzk0OTIyLCJsZWZ0Ijo1Ljg4MjM1MjgyODk3OTQ5Miwid2lkdGgiOjI3LjIzNTI5NDM0MjA0MTAxNiwiaGVpZ2h0IjoxLjcwMzI4MjU5NDY4MDc4NjF9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI3OCJ9fV0%3D.

Table 5: Limitations of Existing Regulatory Frameworks

| Limitation | Description |
| --- | --- |
| Insufficient Standards | National water quality criteria failing to prevent HABs |
| Fragmented Enforcement | Overlapping jurisdictions and weak implementation |
| Inadequate Localization | Top-down approaches lacking site-specific adaptations |
| Governance Lapses | Underfunded monitoring, corruption, and community exclusion |

The "wicked" nature of eutrophication site-specific hydrology, socio-economic variances, and interactive stressors renders uniform top-down strategies ineffective (Foulon et al., 2019; Khorasani & Zhu, 2021)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IsOJdGllbm5lIiwiZmFtaWx5IjoiRm91bG9uIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFsYWluIE4uIiwiZmFtaWx5IjoiUm91c3NlYXUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiR2xlbm4iLCJmYW1pbHkiOiJCZW5veSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJSZWJlY2NhIEwuIiwiZmFtaWx5IjoiTm9ydGgifX1dLCJpZHMiOnsiaXNzbiI6WyIxMjAxLTMwODAiLCIyNDA4LTk0NDMiXSwiaXNibiI6W10sImRvaSI6IjEwLjIxNjYvd3Fyai4yMDE5LjAxMyIsIm1hZyI6IjI5NzM4NjAyODEiLCJvcGVuYWxleCI6IlcyOTczODYwMjgxIn0sInRpdGxlIjoiQSBnbG9iYWwgc2NhbiBvZiBob3cgdGhlIGlzc3VlIG9mIG51dHJpZW50IGxvYWRpbmcgYW5kIGhhcm1mdWwgYWxnYWwgYmxvb21zIGlzIGJlaW5nIGFkZHJlc3NlZCBieSBnb3Zlcm5tZW50cywgbm9uLWdvdmVybm1lbnRhbCBvcmdhbml6YXRpb25zLCBhbmQgdm9sdW50ZWVycyIsImFic3RyYWN0IjoiQWJzdHJhY3QgSGFybWZ1bCBhbGdhbCBibG9vbXMgKEhBQnMpIGluIGFxdWF0aWMgZWNvc3lzdGVtcyBhcmUgb2YgY29uY2VybiB3b3JsZHdpZGUuIFRoaXMgcmV2aWV3IGRlYWxzIHdpdGggaG93IGp1cmlzZGljdGlvbnMgYXJvdW5kIHRoZSB3b3JsZCBhcmUgYWRkcmVzc2luZyB0aGlzIHdhdGVyIHF1YWxpdHkgaXNzdWUgdG8gaW5mb3JtIHJlY29tbWVuZGF0aW9ucyByZWdhcmRpbmcgbnV0cmllbnQgbG9hZGluZyBhbmQgSEFCcyBpbiBNaXNzaXNxdW9pIEJheS1MYWtlIENoYW1wbGFpbiBhbmQgTGFrZSBNZW1waHJlbWFnb2c7IHRyYW5zYm91bmRhcnkgbGFrZXMgbG9jYXRlZCBpbiB0aGUgVVNBIGFuZCBDYW5hZGEgdGhhdCBzdWZmZXIgZnJvbSBzeW1wdG9tcyBvZiBldXRyb3BoaWNhdGlvbi4gQSBnbG9iYWwgc2NhbiBvZiB0aGUgbGl0ZXJhdHVyZSByZXN1bHRlZCBpbiB0aGUgY29uc2lkZXJhdGlvbiBvZiAxMiBjYXNlIHN0dWRpZXMgb2YgbGFyZ2Ugd2F0ZXIgYm9kaWVzIHdpdGhpbiBsYXJnZSB3YXRlcnNoZWRzLCBleGNsdWRpbmcgaW4tbGFrZSBnZW9lbmdpbmVlcmluZyBhcHByb2FjaGVzLiBBbHRob3VnaCBhbGwgb2YgdGhlIHN5c3RlbXMgZXhwZXJpZW5jZSBleGNlc3NpdmUgbnV0cmllbnQgbG9hZGluZywgdGhleSB2YXJ5IGluIHR3byBrZXkgd2F5czogc291cmNlcyBvZiBudXRyaWVudHMgYW5kIG1hbmlmZXN0YXRpb25zIG9mIGV1dHJvcGhpY2F0aW9uIHJhbmdpbmcgZnJvbSBIQUJzLCB0byBsaW1pdGVkIHJlY3JlYXRpb25hbCB1c2VzLCB0byB0aGUgYWRkaXRpb25hbCBjb21wbGV4aXR5IG9mIGludGVybmFsIGxvYWRpbmdzIGFuZCBmaXNoIGtpbGxzLCB1cCB0byBkcmlua2luZyB3YXRlciBzaHV0ZG93bnMuIFRoZSBjYXNlIHN0dWRpZXMgd2VyZSBhbmFseXplZCB3aXRoIHJlc3BlY3QgdG8gZm91ciBjYXRlZ29yaWVzIG9mIGFwcHJvYWNoZXMsIG5hbWVseTogKGkpIHJlZ3VsYXRvcnk7IChpaSkgaW5jZW50aXZlLWJhc2VkOyAoaWlpKSByaXNrIG1pdGlnYXRpb247IGFuZCAoaXYpIG91dHJlYWNoLCBlbmdhZ2VtZW50LCBhbmQgZWR1Y2F0aW9uYWwuIFdlIGZvdW5kIHRoYXQgdGhlIG1hbmFnZW1lbnQgZnJhbWV3b3JrcyBhcmUgYmFzZWQgb24gaW50ZWdyYXRlZCB3YXRlcnNoZWQgbWFuYWdlbWVudCBwbGFubmluZyBhbmQgbmF0aW9uYWwgc3RhbmRhcmRzLiBOYXRpb25hbCB3YXRlciBxdWFsaXR5IHN0YW5kYXJkcywgaG93ZXZlciwgYXJlIG5vdCBzdHJpbmdlbnQgZW5vdWdoIHRvIHByZXZlbnQgSEFCcy4gT3ZlcmFsbCwgaWRlbnRpZmllZCBjYXNlIHN0dWRpZXMgZGlkIG5vdCBzdWNjZXNzZnVsbHkgcmVtZWRpYXRlIEhBQnMsIHRoZXkgc2ltcGx5IG1hbmFnZWQgdGhlbS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjE2Ni93cXJqLjIwMTkuMDEzIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2l3YXBvbmxpbmUuY29tL3dxcmovYXJ0aWNsZS1wZGYvNTUvMS8xLzcwOTI2MS93cXJqYzA1NTAwMDEucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMTktMDktMTciLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA5IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJXYXRlciBRdWFsaXR5IFJlc2VhcmNoIEpvdXJuYWwiLCJpc3N1ZSI6MSwidm9sdW1lIjo1NSwiaW1wYWN0RmFjdG9yIjoiNy44MyJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjEsImxhc3RQYWdlIjoyM319LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJVV0EgUHVibGlzaGluZyJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6ODQsImNpdGVkQnlDb3VudCI6MjB9fSwicXVvdGUiOiJXZSBwcm9wb3NlIHNvbHV0aW9ucyBhbmQgcGF0aHMgZm9yd2FyZCB0byBhIHdpY2tlZCBwcm9ibGVtIHdoaWNoIHNob3VsZCBwcm92ZSBpbnNpZ2h0ZnVsIGZvciBwb2xpY3ktbWFrZXJzIGFuZCBtYW4tIGFnZXJzIGFsaWtlIHRocm91Z2ggdGhlIGZvbGxvd2luZyBmaW5kaW5ncy4gTmF0aW9uYWwgd2F0ZXIgcXVhbGl0eSBzdGFuZGFyZHMgaGF2ZSBub3QgcHJvdmVkIHN0cmluZ2VudCBlbm91Z2ggdG8gcHJlLSB2ZW50IEhBQnMuIElkZW50aWZpZWQgZnJhbWV3b3JrcyBhcmUgYWxsIGJhc2VkIG9uIGludGVncmF0ZWQgd2F0ZXJzaGVkIG1hbmFnZW1lbnQgYW5kIG5hdGlvbmFsIHN0YW5kYXJkcy4gTm8gYWN0dWFsIHN1Y2Nlc3Mgc3RvcmllcyByZWxhdGVkIHRvIHJlbWVkaWF0aW9uIG9mIEhBQnMgd2VyZSBpZGVudGlmaWVkOyB0aGV5IHdlcmUgc2ltcGx5IG1hbmFnZW1lbnQgc3Rvcmllcy4gVGhlIGVmZmljYWN5IG9mIE5QUyBCTVBzIGlzIHF1ZXN0aW9uZWQgdGhyb3VnaG91dCB0aGUgd29ybGQgZHVlIHRvIGluc3VmZmljaWVudCBldmlkZW5jZSB0byBkZXRlcm1pbmUgd2hldGhlciB0aGV5IGNhbiBtaXRpZ2F0ZSBIQUJzLiBNYXJrZXQtYmFzZWQgc3RyYXRlZ2llcyB0byBvZmZzZXQgcG9sbHV0aW9uIG9yIHByb21vdGUgdGFyZ2V0ZWQgQk1QIGFkb3B0aW9uIGFyZSBiZWNvbWluZyBwb3B1bGFyIGFuZCByZXN1bHRpbmcgaW4gY29zdC1lZmZlY3RpdmUgYW5kIGVmZmljaWVudCBhcHByb2FjaGVzLiBDb3N04oCTYmVuZWZpdCBhbmFseXNlcyBtYXkgaW5jcmVhc2UgdGhlIHVzZSBvZiB0ZWNobmljYWwgc29sLSB1dGlvbnMgb3ZlciBjb21wcmVoZW5zaXZlIGFwcHJvYWNoZXMuIEF0IHRoaXMgcG9pbnQsIHRoZXJlIGlzIG5vdCByZWFsbHkgYSB1bmlxdWUgY2FzZSBzdHVkeSB0aGF0IGNvdWxkIGJlIGFwcGxpZWQgdG8gTGFrZSBDaGFtcGxhaW4gYW5kIExha2UgTWVtcGhyZW1hZ29nLiBTcGVjaWZpYyBlbGVtZW50cyBvZiB0aGUgYXBwcm9hY2hlcyBpZGVudGlmaWVkIGluIHRoaXMgcmV2aWV3IGFuZCB1bmktIGZ5aW5nIHRoZW1lcywgaG93ZXZlciwgaG9sZCBwcm9taXNlcy4gVGhpcyBpcyB3aHkgd2UgYWR2b2NhdGUgZm9yIGNvbmR1Y3RpbmcgYSBzeXN0ZW1hdGljIGxpdGVyYXR1cmUgcmV2aWV3IHdpdGggcmVnYXJkIHRvIHRob3NlIHRoZW1lcyBpbiBvcmRlciB0byBkZXZlbG9wIHJlY29tbWVuLSBkYXRpb25zIHRhaWxvcmVkIHRvIHRoZSBMYWtlIE1lbXBocmVtYWdvZyBhbmQgTWlzc2lzcXVvaSBCYXktTGFrZSBDaGFtcGxhaW4gYXJlYXMuIEF0IGxhc3QsIHdlIHdvdWxkIGxpa2UgdG8gYWRkcmVzcyB0aGUgZWxlcGhhbnQtc2l6ZWQgcXVlcy0gdGlvbiBpbiB0aGUgcm9vbS4gV2lsbCBjbGltYXRlIGNoYW5nZSBpbmNyZWFzZSBIQUJzIGZyZXF1ZW5jeT8gSW50ZXJhY3RpdmUgZWZmZWN0cyBvZiBldXRyb3BoaWNhdGlvbiBhbmQgY2xpbWF0ZSBjaGFuZ2Ugb24gaGFybWZ1bCBjeWFub2JhY3RlcmlhbCBibG9vbXMgYXJlIGNvbXBsZXguICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxNSwiYmJveCI6eyJ0b3AiOjg1LjYwNTUxNDUyNjM2NzE5LCJsZWZ0Ijo0OS43OTQ4NzIyODM5MzU1NSwid2lkdGgiOjM5LjcxODY2MjI2MTk2Mjg5LCJoZWlnaHQiOjEuMzI4NjcxMzM2MTc0MDExMn19LHsicGFnZU51bWJlciI6MTUsImJib3giOnsidG9wIjo4Ny4zOTUzNjI4NTQwMDM5LCJsZWZ0Ijo0OS43OTQ4NzIyODM5MzU1NSwid2lkdGgiOjM5LjcwMjcwOTE5Nzk5ODA1LCJoZWlnaHQiOjEuMzI4NjcxMzM2MTc0MDExMn19LHsicGFnZU51bWJlciI6MTUsImJib3giOnsidG9wIjo4OS4xODUyMTExODE2NDA2MiwibGVmdCI6NDkuNzk0ODcyMjgzOTM1NTUsIndpZHRoIjozOS43MDIwOTUwMzE3MzgyOCwiaGVpZ2h0IjoxLjMyODY3MTMzNjE3NDAxMTJ9fSx7InBhZ2VOdW1iZXIiOjE1LCJiYm94Ijp7InRvcCI6OTAuOTc1MDU5NTA5Mjc3MzQsImxlZnQiOjQ5Ljc5NDg3MjI4MzkzNTU1LCJ3aWR0aCI6MzkuNzA0NjIwMzYxMzI4MTI1LCJoZWlnaHQiOjEuMzI4NjcxMzM2MTc0MDExMn19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjoxNC4wMjU5OTA0ODYxNDUwMiwibGVmdCI6MTAuNTAyMzIzMTUwNjM0NzY2LCJ3aWR0aCI6MzkuNzA1OTU1NTA1MzcxMDk0LCJoZWlnaHQiOjEuMzI4NjY5NDI4ODI1Mzc4NH19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjoxNS44MTU4NDE2NzQ4MDQ2ODgsImxlZnQiOjEwLjUwMjMyMzE1MDYzNDc2Niwid2lkdGgiOjM5LjcwMjI0NzYxOTYyODkwNiwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MTcuNjA1NjkxOTA5NzkwMDQsImxlZnQiOjEwLjUwMjE3NjI4NDc5MDAzOSwid2lkdGgiOjM5LjcwMDkyMzkxOTY3NzczNCwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MTkuMzk1NTQ0MDUyMTI0MDIzLCJsZWZ0IjoxMC41MDIxNzYyODQ3OTAwMzksIndpZHRoIjozOS43MDg5MDA0NTE2NjAxNTYsImhlaWdodCI6MS4zMjg2Njk0Mjg4MjUzNzg0fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjIxLjE4NTM5NDI4NzEwOTM3NSwibGVmdCI6MTAuNTAyMTc2Mjg0NzkwMDM5LCJ3aWR0aCI6MzkuNzEwMDgzMDA3ODEyNSwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MjIuOTc1MjQ0NTIyMDk0NzI3LCJsZWZ0IjoxMC41MDIxNzYyODQ3OTAwMzksIndpZHRoIjozOS43MDQwMjkwODMyNTE5NSwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MjQuNzY1MDk2NjY0NDI4NzEsImxlZnQiOjEwLjUwMjE3NjI4NDc5MDAzOSwid2lkdGgiOjM5LjcwMzg4MDMxMDA1ODU5NCwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MjYuNTU0OTQ4ODA2NzYyNjk1LCJsZWZ0IjoxMC41MDIwMjc1MTE1OTY2OCwid2lkdGgiOjM5LjcwMzg4MDMxMDA1ODU5NCwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MjguMzQ0Nzk5MDQxNzQ4MDQ3LCJsZWZ0IjoxMC41MDIwMjc1MTE1OTY2OCwid2lkdGgiOjM5LjcwMDAzODkwOTkxMjExLCJoZWlnaHQiOjEuMzI4NjY5NDI4ODI1Mzc4NH19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjozMC4xMzQ2NDczNjkzODQ3NjYsImxlZnQiOjEwLjUwMjAyNzUxMTU5NjY4LCJ3aWR0aCI6MzkuNzA0NjE2NTQ2NjMwODYsImhlaWdodCI6MS4zMjg2Njk0Mjg4MjUzNzg0fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjMxLjkyNDQ5NzYwNDM3MDExNywibGVmdCI6MTAuNTAxODc3Nzg0NzI5MDA0LCJ3aWR0aCI6MzkuNzA1Nzg3NjU4NjkxNDA2LCJoZWlnaHQiOjEuMzI4NjY5NDI4ODI1Mzc4NH19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjozMy43MTQzNTE2NTQwNTI3MzQsImxlZnQiOjEwLjUwMTcyOTAxMTUzNTY0NSwid2lkdGgiOjM5LjcxOTI0NTkxMDY0NDUzLCJoZWlnaHQiOjEuMzI4NjY3NDAyMjY3NDU2fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjM1LjUwNDIwMzc5NjM4NjcyLCJsZWZ0IjoxMC41MDE3MjkwMTE1MzU2NDUsIndpZHRoIjozOS43MDI3MDkxOTc5OTgwNSwiaGVpZ2h0IjoxLjMyODY2NzQwMjI2NzQ1Nn19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjozNy4yOTQwNTIxMjQwMjM0NCwibGVmdCI6MTAuNTAxNzI5MDExNTM1NjQ1LCJ3aWR0aCI6MzkuNzA1Nzk5MTAyNzgzMiwiaGVpZ2h0IjoxLjMyODY2NzQwMjI2NzQ1Nn19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjozOS4wODM5MDQyNjYzNTc0MiwibGVmdCI6MTAuNTAxNzI1MTk2ODM4Mzc5LCJ3aWR0aCI6MzkuNzA2NTM1MzM5MzU1NDcsImhlaWdodCI6MS4zMjg2Njc0MDIyNjc0NTZ9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6NDAuODczNzU2NDA4NjkxNDA2LCJsZWZ0IjoxMC41MDE3MjcxMDQxODcwMTIsIndpZHRoIjozOS43MjExNTMyNTkyNzczNDQsImhlaWdodCI6MS4zMjg2Njc0MDIyNjc0NTZ9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6NDIuNjYzNjA4NTUxMDI1MzksImxlZnQiOjEwLjUwMTcyNzEwNDE4NzAxMiwid2lkdGgiOjM5LjcwMzEzNjQ0NDA5MTgsImhlaWdodCI6MS4zMjg2Njc0MDIyNjc0NTZ9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6NDQuNDQ2MjA4OTUzODU3NDIsImxlZnQiOjEwLjUwMTcyNzEwNDE4NzAxMiwid2lkdGgiOjM5LjcwMDc3NTE0NjQ4NDM3NSwiaGVpZ2h0IjoxLjMyODY2NzQwMjI2NzQ1Nn19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjo0Ni4yMzYwNjEwOTYxOTE0MDYsImxlZnQiOjEwLjUwMTcyNzEwNDE4NzAxMiwid2lkdGgiOjM5LjcwNzcyNTUyNDkwMjM0NCwiaGVpZ2h0IjoxLjMyODY2NzQwMjI2NzQ1Nn19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjo0OC4wMjU5MTMyMzg1MjUzOSwibGVmdCI6MTAuNTAxNzI3MTA0MTg3MDEyLCJ3aWR0aCI6MjUuOTQxNjY3NTU2NzYyNjk1LCJoZWlnaHQiOjEuMzI4NjY3NDAyMjY3NDU2fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjQ5LjgxNTc2MTU2NjE2MjExLCJsZWZ0IjoxMy4xMjUwMTYyMTI0NjMzNzksIndpZHRoIjozNy4wODExODA1NzI1MDk3NjYsImhlaWdodCI6MS4zMjg2Njc0MDIyNjc0NTZ9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6NTEuNjA1NjEzNzA4NDk2MDk0LCJsZWZ0IjoxMC41MDE3MjcxMDQxODcwMTIsIndpZHRoIjozOS43MDkyMDU2Mjc0NDE0MDYsImhlaWdodCI6MS4zMjg2Njc0MDIyNjc0NTZ9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6NTMuMzk1NDYyMDM2MTMyODEsImxlZnQiOjEwLjUwMTcyNzEwNDE4NzAxMiwid2lkdGgiOjM5LjcxOTIyMzAyMjQ2MDk0LCJoZWlnaHQiOjEuMzI4NjY3NDAyMjY3NDU2fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjU1LjE4NTMxMDM2Mzc2OTUzLCJsZWZ0IjoxMC41MDE3MjcxMDQxODcwMTIsIndpZHRoIjozOS43MTA2NzgxMDA1ODU5NCwiaGVpZ2h0IjoxLjMyODY2NzQwMjI2NzQ1Nn19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjE2In19LHsib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhhbWVkIiwiZmFtaWx5IjoiS2hvcmFzYW5pIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlpoZW5kdW8iLCJmYW1pbHkiOiJaaHUifX1dLCJpZHMiOnsiaXNzbiI6WyIwMDIyLTE2OTQiLCIxODc5LTI3MDciXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMTYvai5qaHlkcm9sLjIwMjEuMTI2ODg2IiwibWFnIjoiMzE3NzA3MTI5OCIsIm9wZW5hbGV4IjoiVzMxNzcwNzEyOTgifSwidGl0bGUiOiJQaG9zcGhvcnVzIHJldGVudGlvbiBpbiBsYWtlczogQSBjcml0aWNhbCByZWFzc2Vzc21lbnQgb2YgaHlwb3RoZXNlcyBhbmQgc3RhdGljIG1vZGVscyIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2ouamh5ZHJvbC4yMDIxLjEyNjg4NiIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9lYXJ0aGFyeGl2Lm9yZy9yZXBvc2l0b3J5L29iamVjdC8yNDgxL2Rvd25sb2FkLzUwNzQvIiwiaXNzdWVkRGF0ZSI6IjIwMjEtMDktMDIiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA4IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJKb3VybmFsIG9mIEh5ZHJvbG9neSIsInZvbHVtZSI6NjAzLCJpbXBhY3RGYWN0b3IiOiI3LjY0In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTI2ODg2LCJsYXN0UGFnZSI6MTI2ODg2fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkVsc2V2aWVyIEJWIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo3NywiY2l0ZWRCeUNvdW50IjoxMn19LCJxdW90ZSI6Ikhvd2V2ZXIsIGh1bWFuIGFjdGl2aXRpZXMgaGF2ZSA0OSBhY2NlbGVyYXRlZCB0aGUgZXV0cm9waGljYXRpb24gcHJvY2VzcyBieSBpbmNyZWFzaW5nIHRoZSBudXRyaWVudHMgZGVsaXZlcnkgdG8gdGhlIGFxdWF0aWMgc3lzdGVtcyA1MCAoTWVrb25uZW4gYW5kIEhvZWtzdHJhLCAyMDE4KS4gVGh1cywgYW50aHJvcG9nZW5pYyBldXRyb3BoaWNhdGlvbiBpcyBvbmUgb2YgdGhlIG1vc3QgaW1wb3J0YW50IDUxIGVsZW1lbnRzIG9mIGZyZXNoIGFuZCBtYXJpbmUgd2F0ZXIgcXVhbGl0eSBkZXRlcmlvcmF0aW9uIChIdSBldCBhbC4sIDIwMTk7IFNtaXRoIGFuZCBTY2hpbmRsZXIsIDUyIDIwMDkpLiBPbmUgZGlyZWN0IGNvbnNlcXVlbmNlIG9mIGFudGhyb3BvZ2VuaWMgZXV0cm9waGljYXRpb24gY29tZXMgaW4gdGhlIGZvcm0gb2YgbWFzc2l2ZSBhbGdhbCA1MyBibG9vbXMgKEdyYW7DqWxpIGV0IGFsLiwgMjAwODsgSGVpc2xlciBldCBhbC4sIDIwMDgpLCB3aGljaCBhcmUgcHJlZGljdGVkIHRvIGJlIGludGVuc2lmaWVkIHVuZGVyIDU0IHdhcm1lciB3YXRlciB0ZW1wZXJhdHVyZXMgYXMgY2xpbWF0ZSBjaGFuZ2VzIChHb2JsZXIsIDIwMjA7IE11a3VuZGFuIGV0IGFsLiwgMjAyMCkuIDU1IEV1dHJvcGhpY2F0aW9uIGlzIGEg4oCcd2lja2Vk4oCdIHByb2JsZW0sIHdoaWNoIGlzIHRoZSBjb25zZXF1ZW5jZSBvZiB2YXJpb3VzIHByb2Nlc3NlcyB0aGF0IG9wZXJhdGUgNTYgY3VtdWxhdGl2ZWx5LiBDb25zaWRlcmluZyB0aGUgdW5pcXVlbmVzcyBvZiBlYWNoIGxha2UgYW5kIGl0cyBzdXJyb3VuZGluZyBhcmVhLCB0aGVyZSBpcyBubyBicm9hZGx5IDU3IGFwcGxpY2FibGUgc2V0IG9mIGJlc3QgbWFuYWdlbWVudCBwcmFjdGljZXMgdGhhdCBjYW4gYmUgYXBwbGllZCBpbiB3YXRlcnNoZWRzIHRvIG1pdGlnYXRlIDU4IHBob3NwaG9ydXMgbG9hZGluZyBhbmQgaXRzIGltcGFjdCBvbiBhbGwgbGFrZXMgKFRob3JudG9uIGV0IGFsLiwgMjAxMykuIEhlbmNlLCBldXRyb3BoaWNhdGlvbiA1OSBtYW5hZ2VtZW50IGFuZCBsYWtlIHJlc3RvcmF0aW9uIG5lZWQgaW50ZWdyYXRlZCBwbGFucyB0aGF0IGFyZSBub3Qgb25seSBzY2llbnRpZmljYWxseSB2YWxpZCBidXQgNjAgYWxzbyBzb2Npby1lY29ub21pY2FsbHkgc2F0aXNmeWluZyAoR2lic29uIGV0IGFsLiwgMjAwMCkuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MzYuMTY4NDM0MTQzMDY2NDA2LCJsZWZ0IjoxMS43Njg2MjgxMjA0MjIzNjMsIndpZHRoIjo3Ni45NjMzOTQxNjUwMzkwNiwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjM2LjcxODQzMzM4MDEyNjk1LCJsZWZ0Ijo2Ljk4MDM5MjQ1NjA1NDY4NzUsIndpZHRoIjoyLjI1MDk1NzQ4OTAxMzY3MiwiaGVpZ2h0IjoxLjM5Mzk0MDQ0ODc2MDk4NjN9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjozOS42NTMyODU5ODAyMjQ2MSwibGVmdCI6Ni45ODAzOTI0NTYwNTQ2ODc1LCJ3aWR0aCI6ODEuNzYxNDI4ODMzMDA3ODEsImhlaWdodCI6Mi4wMjEyMTE2MjQxNDU1MDh9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo0My4xMzgxMzQwMDI2ODU1NSwibGVmdCI6Ni45ODAzOTI0NTYwNTQ2ODc1LCJ3aWR0aCI6ODEuNzU1NTYxODI4NjEzMjgsImhlaWdodCI6Mi4wMjEyMTE2MjQxNDU1MDh9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo0Ni42MjI5ODIwMjUxNDY0ODQsImxlZnQiOjYuOTgwMzkyNDU2MDU0Njg3NSwid2lkdGgiOjgxLjc0MjgwNTQ4MDk1NzAzLCJoZWlnaHQiOjIuMDIxMjExNjI0MTQ1NTA4fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6NTAuMTA3ODMwMDQ3NjA3NDIsImxlZnQiOjYuOTgwMzkyNDU2MDU0Njg3NSwid2lkdGgiOjgxLjc1MjYxNjg4MjMyNDIyLCJoZWlnaHQiOjIuMDIxMjExNjI0MTQ1NTA4fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6NTMuNTk1MjAzMzk5NjU4MiwibGVmdCI6Ni45ODAzOTI0NTYwNTQ2ODc1LCJ3aWR0aCI6ODEuNzQ4NzAzMDAyOTI5NjksImhlaWdodCI6Mi4wMjEyMTE2MjQxNDU1MDh9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo1Ny4wODAwNDc2MDc0MjE4NzUsImxlZnQiOjYuOTgwMzkyNDU2MDU0Njg3NSwid2lkdGgiOjczLjMxMDQ1NTMyMjI2NTYyLCJoZWlnaHQiOjIuMDIxMjExNjI0MTQ1NTA4fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6NjEuNTY0OTAzMjU5Mjc3MzQ0LCJsZWZ0Ijo2Ljk4MDM5MjQ1NjA1NDY4NzUsIndpZHRoIjo4MS43NTc1MTQ5NTM2MTMyOCwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjY1LjA0OTc0MzY1MjM0Mzc1LCJsZWZ0Ijo2Ljk4MDM5MjQ1NjA1NDY4NzUsIndpZHRoIjo4MS43NTk0NzU3MDgwMDc4MSwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjY4LjUzNDU5MTY3NDgwNDY5LCJsZWZ0Ijo2Ljk4MDM5MjQ1NjA1NDY4NzUsIndpZHRoIjo4MS43MDM5MzM3MTU4MjAzMSwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjcyLjAxOTQ1NDk1NjA1NDY5LCJsZWZ0Ijo2Ljk4MDM5MjQ1NjA1NDY4NzUsIndpZHRoIjo4MS43NTA2NTYxMjc5Mjk2OSwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjc1LjUwODA4NzE1ODIwMzEyLCJsZWZ0Ijo2Ljk4MDM5MjQ1NjA1NDY4NzUsIndpZHRoIjo4MS43NTc1MTQ5NTM2MTMyOCwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjc4Ljk5MjkzNTE4MDY2NDA2LCJsZWZ0IjoxMS43Njg2MjgxMjA0MjIzNjMsIndpZHRoIjo3Ni45NzMxOTc5MzcwMTE3MiwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEyNjg4OCJ9fV0%3D. Local implementation fails amid corruption, community exclusion, and data scarcity; for instance, Dal Lake dredging recycles sediments without watershed nutrient curbs (Ji et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkRhb2JpbiIsImZhbWlseSI6IkppIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllhbnhpbmciLCJmYW1pbHkiOiJIYW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTGlhbmdob25nIiwiZmFtaWx5IjoiTG9uZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJYaWFva2FuZyIsImZhbWlseSI6IlhpbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJIdWl5YW4iLCJmYW1pbHkiOiJYdSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTaXFpYW4iLCJmYW1pbHkiOiJRaXUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSmlhbmdodWFpIiwiZmFtaWx5IjoiTWVuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJYaW5neGluZyIsImZhbWlseSI6IlpoYW8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiWWFuYW4iLCJmYW1pbHkiOiJIdWFuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJEZWZ1IiwiZmFtaWx5IjoiTGl1In19XSwiaWRzIjp7Imlzc24iOlsiMjI5Ni03MDFYIl0sImlzYm4iOltdLCJkb2kiOiIxMC4zMzg5L2Zldm8uMjAyMy4xMjk3MDQ3Iiwib3BlbmFsZXgiOiJXNDM5MDYxMDY2OCJ9LCJ0aXRsZSI6Ikh5cG94aWEgYW5kIGl0cyBmZWVkYmFjayByZXNwb25zZSB0byBhbGdhbCBibG9vbXMgYW5kIENINCBlbWlzc2lvbnMgaW4gc3VidHJvcGljYWwgcmVzZXJ2b2lycyIsImFic3RyYWN0IjoiQW50aHJvcG9nZW5pYyBudXRyaWVudCBpbnB1dCBzdGltdWxhdGVzIHN1cmZhY2UgZXV0cm9waGljYXRpb24gYW5kIGFsZ2FsIGJsb29tcyBhbmQgdGh1cyBleGFjZXJiYXRlcyBveHlnZW4gZGVwbGV0aW9uIGFuZCBoeXBveGlhIGluIGFxdWF0aWMgZWNvc3lzdGVtcy4gSG93ZXZlciwgZXhpc3Rpbmcgc3R1ZGllcyBoYXZlIHBhaWQgbGl0dGxlIGF0dGVudGlvbiBvbiB0aGUgdW5kZXJzdGFuZGluZyBvZiB0aGUgZmVlZGJhY2sgcmVsYXRpb25zaGlwcyBhbW9uZyBveHlnZW4gY29uc3VtcHRpb24sIGFsZ2FsIHN1Y2Nlc3Npb24gYW5kIGdyZWVuaG91c2UgZ2FzIGVmZmVjdCBhZnRlciByaXZlciBkYW1taW5nLCBwYXJ0aWN1bGFybHkgaW4gVGhyZWUgR29yZ2VzIFJlc2Vydm9pci4gVGhpcyBmaWVsZCB3b3JrIGludmVzdGlnYXRlZCB0aGUgdGhlcm1hbCBzdHJhdGlmaWNhdGlvbiwgd2F0ZXIgcXVhbGl0eSBhbmQgcGh5dG9wbGFua3RvbiBpbiBmb3VyIHRyaWJ1dGFyaWVzIChQZW5neGkgQmF5IChQWEIpLCBEYW5pbmcgQmF5IChETkIpLCBTaGVubm9uZyBCYXkgKFNOQiksIGFuZCBYaWFuZ3hpIEJheSAoWFhCKSkgb2YgdGhlIFRocmVlIEdvcmdlcyBSZXNlcnZvaXIgKFRHUikgaW4gc3ByaW5nIChBcHJpbCAyNS0yOCwgMjAyMiksIGFuZCBleHBsb3JlZCB0aGUgZHJpdmVycyBvZiBoeXBveGlhIGFuZCBpdHMgZmVlZGJhY2sgcmVzcG9uc2UgdG8gYWxnYWwgYmxvb21zIGFuZCBtZXRoYW5lIChDSCA0ICkgZW1pc3Npb25zLiBUaGUgcmVzdWx0cyBzaG93ZWQgdGhhdCBkdXJpbmcgdGhlIG9ic2VydmF0aW9uIHBlcmlvZCwgRE8sIHRoZXJtYWwgc3RyYXRpZmljYXRpb24sIGFuZCBhbGdhbCBibG9vbXMgd2VyZSBtb3JlIHNldmVyZSBpbiBQWEIuIFRoZSBoaWdoIENoZW1pY2FsIE94eWdlbiBEZW1hbmQgKENPRCBNbiApICgxMS4yNyBtZ%2BKAokwgLTEgKSBhbmQgRGlzc29sdmVkIE9yZ2FuaWMgQ2FyYm9uIChET0MpIGNvbnRlbnQgKDcuNTYgbWfigKJMIC0xICkgaW4gUFhCIHdlcmUgdGhlIG1haW4gY2F1c2VzIG9mIHRoZSBwcm9ncmVzc2l2ZSBoeXBveGlhLiBGdXJ0aGVybW9yZSwgc3Ryb25nZXIgZGVuc2l0eSBzdHJhdGlmaWNhdGlvbiBpbiBQWEIgaW5oaWJpdGVkIHRoZSB2ZXJ0aWNhbCBzdXBwbHkgb2YgRE8gYW5kIGFjY2VsZXJhdGVkIGJvdHRvbSBoeXBveGlhLiBCdXQgdGhlIGRvd25zbG9wZSBkZW5zaXR5IGN1cnJlbnQgcmVwbGVuaXNoZWQgYm90dG9tIERPIGluIHRoZSB1cHBlciByZWFjaGVzIG9mIHRoZSBETkIsIFNOQiwgYW5kIFhYQi4gVGhlIENINCBjb25jZW50cmF0aW9uIHdhcyBnZW5lcmFsbHkgaGlnaGVyIGluIGh5cG94aWEgem9uZSwgZXNwZWNpYWxseSBpbiB3aGVyZSBjeWFub2JhY3RlcmlhIGFyZSB0aGUgZG9taW5hbnQgc3BlY2llcyAoZWcuLCBQWDAzIGFuZCBQWDA0KS4gRGFtIGNvbnN0cnVjdGlvbiBsZWQgdG8gdGhlIHN1Y2Nlc3Npb24gb2YgYWxnYWwtZG9taW5hbnQgc3BlY2llcywgZXhlcnRpbmcgYSBzaWduaWZpY2FudCBpbmZsdWVuY2Ugb24gdGhlIHJpdmVyIGNhcmJvbiBjeWNsZS4gVGhlIGV4cGFuc2lvbiBvZiBjeWFub2JhY3RlcmlhbCBibG9vbXMgYW5kIHRoZSBjb25zZXF1ZW5jZSBvZiBtZXRhYm9saWMgc3Vic3RyYXRlcyBpbmNyZWFzZSBhcmUgZ3JhZHVhbGx5IGluY3JlYXNpbmcgb3h5Z2VuIGRlcHJpdmF0aW9uLCBldmVuIGh5cG94aWEsIGFuZCBlbmhhbmNpbmcgQ0ggNCBwcm9kdWN0aW9uIGluIHJpdmVycy4gVGhlc2UgZmluZGluZ3Mgc3VnZ2VzdCB0aGF0IHRoZSBzdWNjZXNzaW9uIG9mIGFsZ2FsLWRvbWluYW50IHNwZWNpZXMgY2F1c2VkIGJ5IHJpdmVyIGRhbW1pbmcgaGFzIGEgcHJvbm91bmNlZCBwb3NpdGl2ZSBmZWVkYmFjayBlZmZlY3Qgb24gcmVzZXJ2b2lyIGh5cG94aWEgcHJvY2Vzc2VzIGFuZCBncmVlbmhvdXNlIGVmZmVjdC4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMzM4OS9mZXZvLjIwMjMuMTI5NzA0NyIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cuZnJvbnRpZXJzaW4ub3JnL2FydGljbGVzLzEwLjMzODkvZmV2by4yMDIzLjEyOTcwNDcvcGRmP2lzUHVibGlzaGVkVjI9RmFsc2UiLCJpc3N1ZWREYXRlIjoiMjAyNC0wMS0wNSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZyb250aWVycyBpbiBFY29sb2d5IGFuZCBFdm9sdXRpb24iLCJ2b2x1bWUiOjExLCJpbXBhY3RGYWN0b3IiOiI0Ljc4In0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkZyb250aWVycyBNZWRpYSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NDAsImNpdGVkQnlDb3VudCI6NX19LCJxdW90ZSI6Im9nZW5pYyBudXRyaWVudCBpbnB1dCBzdGltdWxhdGVzIHN1cmZhY2UgZXV0cm9waGljYXRpb24gYW5kIGFsZ2FsIGJsb29tcyBhbmQgdGh1cyBleGFjZXJiYXRlcyBveHlnZW4gZGVwbGV0aW9uIGFuZCBoeXBveGlhIGluIGFxdWF0aWMgZWNvc3lzdGVtcy4gSG93ZXZlciwgZXhpc3Rpbmcgc3R1ZGllcyBoYXZlIHBhaWQgbGl0dGxlIGF0dGVudGlvbiBvbiB0aGUgdW5kZXJzdGFuZGluZyBvZiB0aGUgZmVlZGJhY2sgcmVsYXRpb25zaGlwcyBhbW9uZyBveHlnZW4gY29uc3VtcHRpb24sIGFsZ2FsIHN1Y2Nlc3Npb24gYW5kIGdyZWVuaG91c2UgZ2FzIGVmZmVjdCBhZnRlciByaXZlciBkYW1taW5nLCAuLi4gRE8gcGxheXMgYSBrZXkgcm9sZSBpbiB0aGUgYmlvZ2VvY2hlbWljYWwgY3ljbGUgb2Ygc3VyZmFjZSB3YXRlcnMgYW5kIGlzIGEgc2Vuc2l0aXZlIGluZGljYXRvciBvZiBwaHlzaWNhbCBhbmQgYmlvZ2VvY2hlbWljYWwgY2hhbmdlcyBpbiByZXNlcnZvaXIgYXF1YXRpYyBlY29zeXN0ZW1zICggV2V0emVsLCAyMDAxICkuIFdoZW4gd2F0ZXJzIGVudGVyIGFuIGFuYWVyb2JpYyBzdGF0ZSwgYXF1YXRpYyBvcmdhbmlzbXMgZGllIGluIGxhcmdlIG51bWJlcnMgYW5kIHRoZSByZWxlYXNlIG9mIHNlZGltZW50LXJlZHVjaW5nIHBvbGx1dGFudHMgKGUuZy4sIGFtbW9uaWEgbml0cm9nZW4sIC4uLiAyMDIyXSApLCBmdXJ0aGVyIGRldGVyaW9yYXRpbmcgd2F0ZXIgcXVhbGl0eS4gV2l0aCB0aGUgaW5jcmVhc2Ugb2YgcG9sbHV0aW9uIGxvYWRzIGluIHdhdGVyc2hlZHMgYW5kIHRoZSBpbnRlbnNpZmljYXRpb24gb2Ygd2F0ZXIgZXV0cm9waGljYXRpb24sIHdhdGVyIGh5cG94aWEgaGFzIGJlY29tZSBhIHNldmVyZSBnbG9iYWwgZWNvbG9naWNhbCBhbmQgZW52aXJvbm1lbnRhbCBwcm9ibGVtIGluIHJlc2Vydm9pcnMgKCBTdW4gZXTCoGFsLiwgMjAyMSA7IEphaXN3YWwgZXTCoGFsLiwgMjAyMyApLiBGb2N1c2luZyBvbiB0aGUgZHluYW1pYyBjaGFuZ2VzIGluIn1d. Restoration narratives dominate success stories, lacking verifiable HAB reductions (Foulon et al., 2019)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IsOJdGllbm5lIiwiZmFtaWx5IjoiRm91bG9uIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFsYWluIE4uIiwiZmFtaWx5IjoiUm91c3NlYXUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiR2xlbm4iLCJmYW1pbHkiOiJCZW5veSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJSZWJlY2NhIEwuIiwiZmFtaWx5IjoiTm9ydGgifX1dLCJpZHMiOnsiaXNzbiI6WyIxMjAxLTMwODAiLCIyNDA4LTk0NDMiXSwiaXNibiI6W10sImRvaSI6IjEwLjIxNjYvd3Fyai4yMDE5LjAxMyIsIm1hZyI6IjI5NzM4NjAyODEiLCJvcGVuYWxleCI6IlcyOTczODYwMjgxIn0sInRpdGxlIjoiQSBnbG9iYWwgc2NhbiBvZiBob3cgdGhlIGlzc3VlIG9mIG51dHJpZW50IGxvYWRpbmcgYW5kIGhhcm1mdWwgYWxnYWwgYmxvb21zIGlzIGJlaW5nIGFkZHJlc3NlZCBieSBnb3Zlcm5tZW50cywgbm9uLWdvdmVybm1lbnRhbCBvcmdhbml6YXRpb25zLCBhbmQgdm9sdW50ZWVycyIsImFic3RyYWN0IjoiQWJzdHJhY3QgSGFybWZ1bCBhbGdhbCBibG9vbXMgKEhBQnMpIGluIGFxdWF0aWMgZWNvc3lzdGVtcyBhcmUgb2YgY29uY2VybiB3b3JsZHdpZGUuIFRoaXMgcmV2aWV3IGRlYWxzIHdpdGggaG93IGp1cmlzZGljdGlvbnMgYXJvdW5kIHRoZSB3b3JsZCBhcmUgYWRkcmVzc2luZyB0aGlzIHdhdGVyIHF1YWxpdHkgaXNzdWUgdG8gaW5mb3JtIHJlY29tbWVuZGF0aW9ucyByZWdhcmRpbmcgbnV0cmllbnQgbG9hZGluZyBhbmQgSEFCcyBpbiBNaXNzaXNxdW9pIEJheS1MYWtlIENoYW1wbGFpbiBhbmQgTGFrZSBNZW1waHJlbWFnb2c7IHRyYW5zYm91bmRhcnkgbGFrZXMgbG9jYXRlZCBpbiB0aGUgVVNBIGFuZCBDYW5hZGEgdGhhdCBzdWZmZXIgZnJvbSBzeW1wdG9tcyBvZiBldXRyb3BoaWNhdGlvbi4gQSBnbG9iYWwgc2NhbiBvZiB0aGUgbGl0ZXJhdHVyZSByZXN1bHRlZCBpbiB0aGUgY29uc2lkZXJhdGlvbiBvZiAxMiBjYXNlIHN0dWRpZXMgb2YgbGFyZ2Ugd2F0ZXIgYm9kaWVzIHdpdGhpbiBsYXJnZSB3YXRlcnNoZWRzLCBleGNsdWRpbmcgaW4tbGFrZSBnZW9lbmdpbmVlcmluZyBhcHByb2FjaGVzLiBBbHRob3VnaCBhbGwgb2YgdGhlIHN5c3RlbXMgZXhwZXJpZW5jZSBleGNlc3NpdmUgbnV0cmllbnQgbG9hZGluZywgdGhleSB2YXJ5IGluIHR3byBrZXkgd2F5czogc291cmNlcyBvZiBudXRyaWVudHMgYW5kIG1hbmlmZXN0YXRpb25zIG9mIGV1dHJvcGhpY2F0aW9uIHJhbmdpbmcgZnJvbSBIQUJzLCB0byBsaW1pdGVkIHJlY3JlYXRpb25hbCB1c2VzLCB0byB0aGUgYWRkaXRpb25hbCBjb21wbGV4aXR5IG9mIGludGVybmFsIGxvYWRpbmdzIGFuZCBmaXNoIGtpbGxzLCB1cCB0byBkcmlua2luZyB3YXRlciBzaHV0ZG93bnMuIFRoZSBjYXNlIHN0dWRpZXMgd2VyZSBhbmFseXplZCB3aXRoIHJlc3BlY3QgdG8gZm91ciBjYXRlZ29yaWVzIG9mIGFwcHJvYWNoZXMsIG5hbWVseTogKGkpIHJlZ3VsYXRvcnk7IChpaSkgaW5jZW50aXZlLWJhc2VkOyAoaWlpKSByaXNrIG1pdGlnYXRpb247IGFuZCAoaXYpIG91dHJlYWNoLCBlbmdhZ2VtZW50LCBhbmQgZWR1Y2F0aW9uYWwuIFdlIGZvdW5kIHRoYXQgdGhlIG1hbmFnZW1lbnQgZnJhbWV3b3JrcyBhcmUgYmFzZWQgb24gaW50ZWdyYXRlZCB3YXRlcnNoZWQgbWFuYWdlbWVudCBwbGFubmluZyBhbmQgbmF0aW9uYWwgc3RhbmRhcmRzLiBOYXRpb25hbCB3YXRlciBxdWFsaXR5IHN0YW5kYXJkcywgaG93ZXZlciwgYXJlIG5vdCBzdHJpbmdlbnQgZW5vdWdoIHRvIHByZXZlbnQgSEFCcy4gT3ZlcmFsbCwgaWRlbnRpZmllZCBjYXNlIHN0dWRpZXMgZGlkIG5vdCBzdWNjZXNzZnVsbHkgcmVtZWRpYXRlIEhBQnMsIHRoZXkgc2ltcGx5IG1hbmFnZWQgdGhlbS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjE2Ni93cXJqLjIwMTkuMDEzIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2l3YXBvbmxpbmUuY29tL3dxcmovYXJ0aWNsZS1wZGYvNTUvMS8xLzcwOTI2MS93cXJqYzA1NTAwMDEucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMTktMDktMTciLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA5IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJXYXRlciBRdWFsaXR5IFJlc2VhcmNoIEpvdXJuYWwiLCJpc3N1ZSI6MSwidm9sdW1lIjo1NSwiaW1wYWN0RmFjdG9yIjoiNy44MyJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjEsImxhc3RQYWdlIjoyM319LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJVV0EgUHVibGlzaGluZyJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6ODQsImNpdGVkQnlDb3VudCI6MjB9fSwicXVvdGUiOiJXZSBwcm9wb3NlIHNvbHV0aW9ucyBhbmQgcGF0aHMgZm9yd2FyZCB0byBhIHdpY2tlZCBwcm9ibGVtIHdoaWNoIHNob3VsZCBwcm92ZSBpbnNpZ2h0ZnVsIGZvciBwb2xpY3ktbWFrZXJzIGFuZCBtYW4tIGFnZXJzIGFsaWtlIHRocm91Z2ggdGhlIGZvbGxvd2luZyBmaW5kaW5ncy4gTmF0aW9uYWwgd2F0ZXIgcXVhbGl0eSBzdGFuZGFyZHMgaGF2ZSBub3QgcHJvdmVkIHN0cmluZ2VudCBlbm91Z2ggdG8gcHJlLSB2ZW50IEhBQnMuIElkZW50aWZpZWQgZnJhbWV3b3JrcyBhcmUgYWxsIGJhc2VkIG9uIGludGVncmF0ZWQgd2F0ZXJzaGVkIG1hbmFnZW1lbnQgYW5kIG5hdGlvbmFsIHN0YW5kYXJkcy4gTm8gYWN0dWFsIHN1Y2Nlc3Mgc3RvcmllcyByZWxhdGVkIHRvIHJlbWVkaWF0aW9uIG9mIEhBQnMgd2VyZSBpZGVudGlmaWVkOyB0aGV5IHdlcmUgc2ltcGx5IG1hbmFnZW1lbnQgc3Rvcmllcy4gVGhlIGVmZmljYWN5IG9mIE5QUyBCTVBzIGlzIHF1ZXN0aW9uZWQgdGhyb3VnaG91dCB0aGUgd29ybGQgZHVlIHRvIGluc3VmZmljaWVudCBldmlkZW5jZSB0byBkZXRlcm1pbmUgd2hldGhlciB0aGV5IGNhbiBtaXRpZ2F0ZSBIQUJzLiBNYXJrZXQtYmFzZWQgc3RyYXRlZ2llcyB0byBvZmZzZXQgcG9sbHV0aW9uIG9yIHByb21vdGUgdGFyZ2V0ZWQgQk1QIGFkb3B0aW9uIGFyZSBiZWNvbWluZyBwb3B1bGFyIGFuZCByZXN1bHRpbmcgaW4gY29zdC1lZmZlY3RpdmUgYW5kIGVmZmljaWVudCBhcHByb2FjaGVzLiBDb3N04oCTYmVuZWZpdCBhbmFseXNlcyBtYXkgaW5jcmVhc2UgdGhlIHVzZSBvZiB0ZWNobmljYWwgc29sLSB1dGlvbnMgb3ZlciBjb21wcmVoZW5zaXZlIGFwcHJvYWNoZXMuIEF0IHRoaXMgcG9pbnQsIHRoZXJlIGlzIG5vdCByZWFsbHkgYSB1bmlxdWUgY2FzZSBzdHVkeSB0aGF0IGNvdWxkIGJlIGFwcGxpZWQgdG8gTGFrZSBDaGFtcGxhaW4gYW5kIExha2UgTWVtcGhyZW1hZ29nLiBTcGVjaWZpYyBlbGVtZW50cyBvZiB0aGUgYXBwcm9hY2hlcyBpZGVudGlmaWVkIGluIHRoaXMgcmV2aWV3IGFuZCB1bmktIGZ5aW5nIHRoZW1lcywgaG93ZXZlciwgaG9sZCBwcm9taXNlcy4gVGhpcyBpcyB3aHkgd2UgYWR2b2NhdGUgZm9yIGNvbmR1Y3RpbmcgYSBzeXN0ZW1hdGljIGxpdGVyYXR1cmUgcmV2aWV3IHdpdGggcmVnYXJkIHRvIHRob3NlIHRoZW1lcyBpbiBvcmRlciB0byBkZXZlbG9wIHJlY29tbWVuLSBkYXRpb25zIHRhaWxvcmVkIHRvIHRoZSBMYWtlIE1lbXBocmVtYWdvZyBhbmQgTWlzc2lzcXVvaSBCYXktTGFrZSBDaGFtcGxhaW4gYXJlYXMuIEF0IGxhc3QsIHdlIHdvdWxkIGxpa2UgdG8gYWRkcmVzcyB0aGUgZWxlcGhhbnQtc2l6ZWQgcXVlcy0gdGlvbiBpbiB0aGUgcm9vbS4gV2lsbCBjbGltYXRlIGNoYW5nZSBpbmNyZWFzZSBIQUJzIGZyZXF1ZW5jeT8gSW50ZXJhY3RpdmUgZWZmZWN0cyBvZiBldXRyb3BoaWNhdGlvbiBhbmQgY2xpbWF0ZSBjaGFuZ2Ugb24gaGFybWZ1bCBjeWFub2JhY3RlcmlhbCBibG9vbXMgYXJlIGNvbXBsZXguICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxNSwiYmJveCI6eyJ0b3AiOjg1LjYwNTUxNDUyNjM2NzE5LCJsZWZ0Ijo0OS43OTQ4NzIyODM5MzU1NSwid2lkdGgiOjM5LjcxODY2MjI2MTk2Mjg5LCJoZWlnaHQiOjEuMzI4NjcxMzM2MTc0MDExMn19LHsicGFnZU51bWJlciI6MTUsImJib3giOnsidG9wIjo4Ny4zOTUzNjI4NTQwMDM5LCJsZWZ0Ijo0OS43OTQ4NzIyODM5MzU1NSwid2lkdGgiOjM5LjcwMjcwOTE5Nzk5ODA1LCJoZWlnaHQiOjEuMzI4NjcxMzM2MTc0MDExMn19LHsicGFnZU51bWJlciI6MTUsImJib3giOnsidG9wIjo4OS4xODUyMTExODE2NDA2MiwibGVmdCI6NDkuNzk0ODcyMjgzOTM1NTUsIndpZHRoIjozOS43MDIwOTUwMzE3MzgyOCwiaGVpZ2h0IjoxLjMyODY3MTMzNjE3NDAxMTJ9fSx7InBhZ2VOdW1iZXIiOjE1LCJiYm94Ijp7InRvcCI6OTAuOTc1MDU5NTA5Mjc3MzQsImxlZnQiOjQ5Ljc5NDg3MjI4MzkzNTU1LCJ3aWR0aCI6MzkuNzA0NjIwMzYxMzI4MTI1LCJoZWlnaHQiOjEuMzI4NjcxMzM2MTc0MDExMn19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjoxNC4wMjU5OTA0ODYxNDUwMiwibGVmdCI6MTAuNTAyMzIzMTUwNjM0NzY2LCJ3aWR0aCI6MzkuNzA1OTU1NTA1MzcxMDk0LCJoZWlnaHQiOjEuMzI4NjY5NDI4ODI1Mzc4NH19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjoxNS44MTU4NDE2NzQ4MDQ2ODgsImxlZnQiOjEwLjUwMjMyMzE1MDYzNDc2Niwid2lkdGgiOjM5LjcwMjI0NzYxOTYyODkwNiwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MTcuNjA1NjkxOTA5NzkwMDQsImxlZnQiOjEwLjUwMjE3NjI4NDc5MDAzOSwid2lkdGgiOjM5LjcwMDkyMzkxOTY3NzczNCwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MTkuMzk1NTQ0MDUyMTI0MDIzLCJsZWZ0IjoxMC41MDIxNzYyODQ3OTAwMzksIndpZHRoIjozOS43MDg5MDA0NTE2NjAxNTYsImhlaWdodCI6MS4zMjg2Njk0Mjg4MjUzNzg0fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjIxLjE4NTM5NDI4NzEwOTM3NSwibGVmdCI6MTAuNTAyMTc2Mjg0NzkwMDM5LCJ3aWR0aCI6MzkuNzEwMDgzMDA3ODEyNSwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MjIuOTc1MjQ0NTIyMDk0NzI3LCJsZWZ0IjoxMC41MDIxNzYyODQ3OTAwMzksIndpZHRoIjozOS43MDQwMjkwODMyNTE5NSwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MjQuNzY1MDk2NjY0NDI4NzEsImxlZnQiOjEwLjUwMjE3NjI4NDc5MDAzOSwid2lkdGgiOjM5LjcwMzg4MDMxMDA1ODU5NCwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MjYuNTU0OTQ4ODA2NzYyNjk1LCJsZWZ0IjoxMC41MDIwMjc1MTE1OTY2OCwid2lkdGgiOjM5LjcwMzg4MDMxMDA1ODU5NCwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MjguMzQ0Nzk5MDQxNzQ4MDQ3LCJsZWZ0IjoxMC41MDIwMjc1MTE1OTY2OCwid2lkdGgiOjM5LjcwMDAzODkwOTkxMjExLCJoZWlnaHQiOjEuMzI4NjY5NDI4ODI1Mzc4NH19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjozMC4xMzQ2NDczNjkzODQ3NjYsImxlZnQiOjEwLjUwMjAyNzUxMTU5NjY4LCJ3aWR0aCI6MzkuNzA0NjE2NTQ2NjMwODYsImhlaWdodCI6MS4zMjg2Njk0Mjg4MjUzNzg0fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjMxLjkyNDQ5NzYwNDM3MDExNywibGVmdCI6MTAuNTAxODc3Nzg0NzI5MDA0LCJ3aWR0aCI6MzkuNzA1Nzg3NjU4NjkxNDA2LCJoZWlnaHQiOjEuMzI4NjY5NDI4ODI1Mzc4NH19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjozMy43MTQzNTE2NTQwNTI3MzQsImxlZnQiOjEwLjUwMTcyOTAxMTUzNTY0NSwid2lkdGgiOjM5LjcxOTI0NTkxMDY0NDUzLCJoZWlnaHQiOjEuMzI4NjY3NDAyMjY3NDU2fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjM1LjUwNDIwMzc5NjM4NjcyLCJsZWZ0IjoxMC41MDE3MjkwMTE1MzU2NDUsIndpZHRoIjozOS43MDI3MDkxOTc5OTgwNSwiaGVpZ2h0IjoxLjMyODY2NzQwMjI2NzQ1Nn19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjozNy4yOTQwNTIxMjQwMjM0NCwibGVmdCI6MTAuNTAxNzI5MDExNTM1NjQ1LCJ3aWR0aCI6MzkuNzA1Nzk5MTAyNzgzMiwiaGVpZ2h0IjoxLjMyODY2NzQwMjI2NzQ1Nn19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjozOS4wODM5MDQyNjYzNTc0MiwibGVmdCI6MTAuNTAxNzI1MTk2ODM4Mzc5LCJ3aWR0aCI6MzkuNzA2NTM1MzM5MzU1NDcsImhlaWdodCI6MS4zMjg2Njc0MDIyNjc0NTZ9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6NDAuODczNzU2NDA4NjkxNDA2LCJsZWZ0IjoxMC41MDE3MjcxMDQxODcwMTIsIndpZHRoIjozOS43MjExNTMyNTkyNzczNDQsImhlaWdodCI6MS4zMjg2Njc0MDIyNjc0NTZ9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6NDIuNjYzNjA4NTUxMDI1MzksImxlZnQiOjEwLjUwMTcyNzEwNDE4NzAxMiwid2lkdGgiOjM5LjcwMzEzNjQ0NDA5MTgsImhlaWdodCI6MS4zMjg2Njc0MDIyNjc0NTZ9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6NDQuNDQ2MjA4OTUzODU3NDIsImxlZnQiOjEwLjUwMTcyNzEwNDE4NzAxMiwid2lkdGgiOjM5LjcwMDc3NTE0NjQ4NDM3NSwiaGVpZ2h0IjoxLjMyODY2NzQwMjI2NzQ1Nn19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjo0Ni4yMzYwNjEwOTYxOTE0MDYsImxlZnQiOjEwLjUwMTcyNzEwNDE4NzAxMiwid2lkdGgiOjM5LjcwNzcyNTUyNDkwMjM0NCwiaGVpZ2h0IjoxLjMyODY2NzQwMjI2NzQ1Nn19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjo0OC4wMjU5MTMyMzg1MjUzOSwibGVmdCI6MTAuNTAxNzI3MTA0MTg3MDEyLCJ3aWR0aCI6MjUuOTQxNjY3NTU2NzYyNjk1LCJoZWlnaHQiOjEuMzI4NjY3NDAyMjY3NDU2fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjQ5LjgxNTc2MTU2NjE2MjExLCJsZWZ0IjoxMy4xMjUwMTYyMTI0NjMzNzksIndpZHRoIjozNy4wODExODA1NzI1MDk3NjYsImhlaWdodCI6MS4zMjg2Njc0MDIyNjc0NTZ9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6NTEuNjA1NjEzNzA4NDk2MDk0LCJsZWZ0IjoxMC41MDE3MjcxMDQxODcwMTIsIndpZHRoIjozOS43MDkyMDU2Mjc0NDE0MDYsImhlaWdodCI6MS4zMjg2Njc0MDIyNjc0NTZ9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6NTMuMzk1NDYyMDM2MTMyODEsImxlZnQiOjEwLjUwMTcyNzEwNDE4NzAxMiwid2lkdGgiOjM5LjcxOTIyMzAyMjQ2MDk0LCJoZWlnaHQiOjEuMzI4NjY3NDAyMjY3NDU2fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjU1LjE4NTMxMDM2Mzc2OTUzLCJsZWZ0IjoxMC41MDE3MjcxMDQxODcwMTIsIndpZHRoIjozOS43MTA2NzgxMDA1ODU5NCwiaGVpZ2h0IjoxLjMyODY2NzQwMjI2NzQ1Nn19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjE2In19XQ%3D%3D. Real estate devaluation and tourism slumps (e.g., 50% occupancy drop in Srinagar) amplify socio-economic tolls, unmitigated by centralized funding (Devlin et al., 2020; Sampat et al., 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjEsInN1YnR5cGUiOnt9LCJjb250cmlidXRvcnMiOlt7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBZGFtIFQuIiwiZmFtaWx5IjoiRGV2bGluIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkppYXlpIiwiZmFtaWx5IjoiUGFuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik0uIE1hbmp1ciIsImZhbWlseSI6IlNoYWgifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQWxpeXUiLCJmYW1pbHkiOiJOdWh1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Iktlbm5ldGggVy4iLCJmYW1pbHkiOiJDdW1taW5zIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkNsYXVkaW8gVmFsZG92aW5vcyIsImZhbWlseSI6IlphcmdlcyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQYWJsbyIsImZhbWlseSI6IkZpZXJybyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJWaXZpYW5hIiwiZmFtaWx5IjoiT2xtb3MifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQWJkdWwtUmFobWFuIiwiZmFtaWx5IjoiRGlyaXN1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkpvaG4gT3ZpZSIsImZhbWlseSI6Ik9sb211a29ybyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJOYXRpdmlkYWRlIiwiZmFtaWx5IjoiVmllaXJhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFuYSIsImZhbWlseSI6IlNvYXJlcyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJDYXJsb3MiLCJmYW1pbHkiOiJQaW5oZWlybyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNYXR0aGV3IEYuIiwiZmFtaWx5IjoiQmluZ2hhbSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJKYXNvbiIsImZhbWlseSI6Iktpbm5lbGwifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiV3UiLCJmYW1pbHkiOiJCbyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJKaW5tdSIsImZhbWlseSI6IlpoYW5nIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllpbmRpIiwiZmFtaWx5IjoiWmhhbyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJIdW1iZXJ0byBBbHZlcyIsImZhbWlseSI6IkJhcmJvc2EifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiRnJhbmtsaW4iLCJmYW1pbHkiOiJQYXJlZGVz4oCQVHJlam8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiVCBMYWtzaG1pIiwiZmFtaWx5IjoiS3VtYXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiS2lyYW4iLCJmYW1pbHkiOiJUaGFrdXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQ2F0YXJpbmEiLCJmYW1pbHkiOiJCdXJpdGkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2NvdHQgQS4iLCJmYW1pbHkiOiJXZWxscyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJPc2NhciBPbW9uZGkiLCJmYW1pbHkiOiJEb25kZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJDYXJlbiBOYXZhbGlhIiwiZmFtaWx5IjoiQXRhbGl0c2EifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiRC4iLCJmYW1pbHkiOiJTYW5pIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNhbXBzb24iLCJmYW1pbHkiOiJBaG1lZCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBbWFuaW5nIiwiZmFtaWx5IjoiS3dha3UifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiR2V2b3JnIiwiZmFtaWx5IjoiQWRqZWkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2F0YXJ1cGEiLCJmYW1pbHkiOiJTaW1vbnlhbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTbmlnZGhhIiwiZmFtaWx5IjoiR2hvc2gifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUHJhc2FubmEiLCJmYW1pbHkiOiJDaGF0dGVyamVlIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNoaXZhIiwiZmFtaWx5IjoiR2hvcmEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQmVsZGEiLCJmYW1pbHkiOiJQcmFzYWQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiS2V0aGxhdGxvZ2lsZSIsImZhbWlseSI6Ik1vc2VwZWxlIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ill1c3VmIiwiZmFtaWx5IjoiU3VsZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJVaXLDoSIsImZhbWlseSI6Ik9saXZlaXJhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1hcmlhIEpvw6NvIiwiZmFtaWx5IjoiVmllaXJhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkZsZXVyIiwiZmFtaWx5IjoiRGUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTWluIiwiZmFtaWx5IjoiTWF4In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikdldm9yZyIsImZhbWlseSI6IlNpbW9ueWFuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNhdGFydXBhIiwiZmFtaWx5IjoiR2hvc2gifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU25pZ2RoYSIsImZhbWlseSI6IkNoYXR0ZXJqZWUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUHJhc2FubmEiLCJmYW1pbHkiOiJQcmFzYWQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2F0YXJ1cGEiLCJmYW1pbHkiOiJHaG9zaCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJHaG9yYSIsImZhbWlseSI6IlByYXNhZCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQcmFzYW5uYSIsImZhbWlseSI6IlBhbCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLd2FrdSIsImZhbWlseSI6IkFnb2R6byJ9fV0sImlkcyI6eyJpc3NuIjpbXSwiaXNibiI6W10sImRvaSI6IjEwLjU3NzIvaW50ZWNob3Blbi44NzQ2MyIsIm1hZyI6IjMxMjg1ODc0NjkiLCJvcGVuYWxleCI6IlczMTI4NTg3NDY5In0sInRpdGxlIjoiSW5sYW5kIFdhdGVycyAtIER5bmFtaWNzIGFuZCBFY29sb2d5IiwiYWJzdHJhY3QiOiJJbmxhbmQgd2F0ZXJzLCBsYWtlcywgcml2ZXJzLCBhbmQgdGhlaXIgY29ubmVjdGVkIHdldGxhbmRzIGFyZSB0aGUgbW9zdCBpbXBvcnRhbnQgYW5kIHRoZSBtb3N0IHZ1bG5lcmFibGUgc291cmNlcyBvZiBmcmVzaHdhdGVyIG9uIHRoZSBwbGFuZXQuIFRoZSBlY29sb2d5IG9mIHRoZXNlIHN5c3RlbXMgaW5jbHVkZXMgYmlvbG9neSBhcyB3ZWxsIGFzIGh1bWFuIHBvcHVsYXRpb25zIGFuZCBjaXZpbGl6YXRpb24uIElubGFuZCB3YXRlcnMgYW5kIHdldGxhbmRzIGFyZSBoaWdobHkgc3VzY2VwdGlibGUgdG8gY2hlbWljYWwgYW5kIGJpb2xvZ2ljYWwgcG9sbHV0YW50cyBmcm9tIG5hdHVyYWwgb3IgaHVtYW4gc291cmNlcywgY2hhbmdlcyBpbiB3YXRlcnNoZWQgZHluYW1pY3MgZHVlIHRvIHRoZSBlc3RhYmxpc2htZW50IG9mIGRhbXMgYW5kIHJlc2Vydm9pcnMsIGFuZCBsYW5kIHVzZSBjaGFuZ2VzIGZyb20gYWdyaWN1bHR1cmUgYW5kIGluZHVzdHJ5LiBUaGlzIGJvb2sgcHJvdmlkZXMgYSBjb21wcmVoZW5zaXZlIHJldmlldyBvZiBpc3N1ZXMgaW52b2x2aW5nIGlubGFuZCB3YXRlcnMgYW5kIGRpc2N1c3NlcyBtYW55IHdvcmxkd2lkZSBpbmxhbmQgd2F0ZXIgc3lzdGVtcy4gVGhlIG1haW4gdG9waWNzIG9mIHRoaXMgdGV4dCBhcmUgd2F0ZXIgcXVhbGl0eSBpbnZlc3RpZ2F0aW9uLCBhbmFseXNlcyBvZiB0aGUgZWNvbG9neSBvZiBpbmxhbmQgd2F0ZXIgc3lzdGVtcywgcmVtb3RlIHNlbnNpbmcgb2JzZXJ2YXRpb24gYW5kIG51bWVyaWNhbCBtb2RlbGluZyBtZXRob2RzLCBhbmQgYmlvZGl2ZXJzaXR5IGludmVzdGlnYXRpb25zLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC41NzcyL2ludGVjaG9wZW4uODc0NjMiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vbXRzLmludGVjaG9wZW4uY29tL3N0b3JhZ2UvYm9va3MvOTY2MC9hdXRob3JzX2Jvb2svYXV0aG9yc19ib29rLnBkZiIsImlzc3VlZERhdGUiOiIyMDIwLTA0LTAxIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiSW50ZWNoT3BlbiBlQm9va3MiLCJpbXBhY3RGYWN0b3IiOiIxLjAxIn0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkludGVjaE9wZW4ifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjMzOCwiY2l0ZWRCeUNvdW50IjoyN319LCJxdW90ZSI6IkFkZGl0aW9uYWxseSwgc2luY2UgYWZmZWN0ZWQgYnVzaW5lc3Nlc%2BKAmSB2YWx1ZXMgIGFyZSBtb3N0IGxpa2VseSB0aWVkIHRvIHRoZWlyIGFzc2V0cyBhbmQgdGhlIHJlYWwgZXN0YXRlIHRoZXkgb2NjdXB5IChmb3IgZXhhbXBsZSwgIGEgbWFyaW5hIGlzIG5vdCBlYXNpbHkgY29udmVydGVkIHRvIHNvbWUgb3RoZXIgdXNlKSwgb24tZ29pbmcgYmFsYW5jZSBzaGVldCBlZmZlY3RzICB3b3VsZCB1bHRpbWF0ZWx5IGxlYWQgdG8gcmVkdWN0aW9ucyBpbiBjb21tZXJjaWFsIHJlYWwgZXN0YXRlIHZhbHVlcy4gVGhlcmUgYXJlIG1hbnkgY2hhbGxlbmdlcyB0byB1bmRlcnN0YW5kaW5nIHRoZSBpbXBsaWNhdGlvbnMgb2YgY2hhbmdlcyBpbiAgdG91cmlzbSBmcm9tIEhBQnMuIFRoZSBjbGVhcmVzdCBjaGFsbGVuZ2Ugb2JzdHJ1Y3RpbmcgYSBwcmVjaXNlIGFuYWx5c2lzIG9mIHRoZXNlICBpbXBhY3RzIGlzIGEgbGFjayBvZiBkYXRhIGVpdGhlciBvbiB0aGUgYW1vdW50IG9mIHRvdXJpc20gYXQgcmlzayBvciB0aGUgc3BlY2lmaWMgIGltcGFjdCBvZiBIQUJzIG9uIHRvdXJpc20uIEZvciBleGFtcGxlLCB3aGlsZSBjb3VudHktbGV2ZWwgZGF0YSBleGlzdHMgZm9yIHRvdGFsICBleHBlbmRpdHVyZXMgb24gdG91cmlzbSwgdGhpcyBpbmNsdWRlcyB0b3VyaXNtIHdoaWNoIHdvdWxkIG5vdCBiZSBpbnRlcnJ1cHRlZCBieSAgSEFCcyBvciBvdGhlciBkaXNjb3VyYWdpbmcgZmFjdG9ycy4gQW4gYWRkaXRpb25hbCBjaGFsbGVuZ2UgcmVsYXRlcyB0byB0aGUgZGlzdGluY3Rpb24gYmV0d2VlbiBlY29ub21pYyBiZW5lZml0cyAgKHdpbGxpbmduZXNzIHRvIHBheSkgYW5kIGVjb25vbWljIGltcGFjdHMgKGV4cGVuZGl0dXJlcyksIGFuZCB0aGUgbWVhc3VyZS1tZW50IG9mIHRoZSBlY29ub21pYyBiZW5lZml0cyB0aGF0IGFyaXNlIGZyb20gZWNvbm9taWMgaW1wYWN0cyAocHJvZml0cykuIEZvciAgZXhhbXBsZSwgY29uc2lkZXIgYSByZXN0YXVyYW50IG93bmVyIHdobyBsb3NlcyAkMTAsMDAwIGluIHJldmVudWUgYmVjYXVzZSBvZiBhICBIQUIuIFRoZSBvd25lcuKAmXMgd2lsbGluZ25lc3MgdG8gcGF5IHRvIHJlY292ZXIgdGhhdCByZXZlbnVlOyBpcyAocm91Z2hseSBzcGVha2luZykgIHRoZSBsb3N0IHByb2ZpdCBvbiB0aGF0IHJldmVudWUuIFRoaXMgaXMgbW9yZSBkaWZmaWN1bHQgdG8gaWRlbnRpZnkgdGhhbiBsb3N0IHJldmVudWUuICAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6ODksImJib3giOnsidG9wIjo1OS40NDI4NjcyNzkwNTI3MzQsImxlZnQiOjE0LjcyMjg1MzY2MDU4MzQ5Niwid2lkdGgiOjY4Ljc0ODAwMTA5ODYzMjgxLCJoZWlnaHQiOjEuOTA1MDAwODA1ODU0Nzk3NH19LHsicGFnZU51bWJlciI6ODksImJib3giOnsidG9wIjo2MS4wNzEwNzU0Mzk0NTMxMjUsImxlZnQiOjE0LjcyMjg1MzY2MDU4MzQ5Niwid2lkdGgiOjY2LjM5NjM0NzA0NTg5ODQ0LCJoZWlnaHQiOjEuOTA1MDAwODA1ODU0Nzk3NH19LHsicGFnZU51bWJlciI6ODksImJib3giOnsidG9wIjo2Mi42OTkyNzU5NzA0NTg5ODQsImxlZnQiOjE0LjcyMjg1MzY2MDU4MzQ5Niwid2lkdGgiOjY5LjI2OTMwOTk5NzU1ODYsImhlaWdodCI6MS45MDUwMDA4MDU4NTQ3OTc0fX0seyJwYWdlTnVtYmVyIjo4OSwiYmJveCI6eyJ0b3AiOjY0LjMyNzQ4NDEzMDg1OTM4LCJsZWZ0IjoxNC43MjI4NTM2NjA1ODM0OTYsIndpZHRoIjo1Ny40MDA1MDUwNjU5MTc5NywiaGVpZ2h0IjoxLjkwNTAwMDgwNTg1NDc5NzR9fSx7InBhZ2VOdW1iZXIiOjg5LCJiYm94Ijp7InRvcCI6NjUuOTU1Njg4NDc2NTYyNSwibGVmdCI6MTcuNSwid2lkdGgiOjYzLjA5NTY5OTMxMDMwMjczNCwiaGVpZ2h0IjoxLjkwNTAwMDgwNTg1NDc5NzR9fSx7InBhZ2VOdW1iZXIiOjg5LCJiYm94Ijp7InRvcCI6NjcuNTgzODkyODIyMjY1NjIsImxlZnQiOjE0LjcyMjg1MzY2MDU4MzQ5Niwid2lkdGgiOjY4LjUyMjU5ODI2NjYwMTU2LCJoZWlnaHQiOjEuOTA0OTk2NjMzNTI5NjYzfX0seyJwYWdlTnVtYmVyIjo4OSwiYmJveCI6eyJ0b3AiOjY5LjIxMjA4OTUzODU3NDIyLCJsZWZ0IjoxNC43MjI4NTM2NjA1ODM0OTYsIndpZHRoIjo2NS40MTIyNzcyMjE2Nzk2OSwiaGVpZ2h0IjoxLjkwNTAwMDgwNTg1NDc5NzR9fSx7InBhZ2VOdW1iZXIiOjg5LCJiYm94Ijp7InRvcCI6NzAuODQwMjkzODg0Mjc3MzQsImxlZnQiOjE0LjcyMjg1MzY2MDU4MzQ5Niwid2lkdGgiOjY3Ljc3MDAxOTUzMTI1LCJoZWlnaHQiOjEuOTA1MDA1MDk3Mzg5MjIxMn19LHsicGFnZU51bWJlciI6ODksImJib3giOnsidG9wIjo3Mi40Njg0OTgyMjk5ODA0NywibGVmdCI6MTQuNzIyODUzNjYwNTgzNDk2LCJ3aWR0aCI6NjkuMzg2MzI5NjUwODc4OSwiaGVpZ2h0IjoxLjkwNTAwNTA5NzM4OTIyMTJ9fSx7InBhZ2VOdW1iZXIiOjg5LCJiYm94Ijp7InRvcCI6NzQuMDk2NzAyNTc1NjgzNiwibGVmdCI6MTQuNzIyODUzNjYwNTgzNDk2LCJ3aWR0aCI6MzAuMTM0ODgwMDY1OTE3OTcsImhlaWdodCI6MS45MDUwMDUwOTczODkyMjEyfX0seyJwYWdlTnVtYmVyIjo4OSwiYmJveCI6eyJ0b3AiOjc1LjcyNDkxNDU1MDc4MTI1LCJsZWZ0IjoxNy41LCJ3aWR0aCI6NjQuNDEwNzk3MTE5MTQwNjIsImhlaWdodCI6MS45MDUwMDUwOTczODkyMjEyfX0seyJwYWdlTnVtYmVyIjo4OSwiYmJveCI6eyJ0b3AiOjc3LjM1MzExMTI2NzA4OTg0LCJsZWZ0IjoxNC43MjI4NTM2NjA1ODM0OTYsIndpZHRoIjo2NC44NTU2OTAwMDI0NDE0LCJoZWlnaHQiOjEuOTA1MDA1MDk3Mzg5MjIxMn19LHsicGFnZU51bWJlciI6ODksImJib3giOnsidG9wIjo3OC45ODEzMTU2MTI3OTI5NywibGVmdCI6MTQuNzIyODUzNjYwNTgzNDk2LCJ3aWR0aCI6NjYuMDE5ODUxNjg0NTcwMzEsImhlaWdodCI6MS45MDUwMDUwOTczODkyMjEyfX0seyJwYWdlTnVtYmVyIjo4OSwiYmJveCI6eyJ0b3AiOjgwLjYwOTUyNzU4Nzg5MDYyLCJsZWZ0IjoxNC43MjI4NTM2NjA1ODM0OTYsIndpZHRoIjo2Ny43ODc2NDM0MzI2MTcxOSwiaGVpZ2h0IjoxLjkwNTAwNTA5NzM4OTIyMTJ9fSx7InBhZ2VOdW1iZXIiOjg5LCJiYm94Ijp7InRvcCI6ODIuMjM3NzI0MzA0MTk5MjIsImxlZnQiOjE0LjcyMjg1MzY2MDU4MzQ5Niwid2lkdGgiOjY5LjczOTY5MjY4Nzk4ODI4LCJoZWlnaHQiOjEuOTA1MDA1MDk3Mzg5MjIxMn19LHsicGFnZU51bWJlciI6ODksImJib3giOnsidG9wIjo4My44NjU5MzYyNzkyOTY4OCwibGVmdCI6MTQuNzIyODUzNjYwNTgzNDk2LCJ3aWR0aCI6NjguNDg3MTI5MjExNDI1NzgsImhlaWdodCI6MS45MDUwMDUwOTczODkyMjEyfX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiOTAifX0seyJvcmlnaW4iOjUsIndvcmsiOnsidHlwZSI6OCwic3VidHlwZSI6eyJvbmVvZktpbmQiOiJhcnRpY2xlVHlwZSIsImFydGljbGVUeXBlIjoxfSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQXBvb3J2YSBNLiIsImZhbWlseSI6IlNhbXBhdCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBbmRyZWEiLCJmYW1pbHkiOiJIaWNrcyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJHZXJhcmRvIEouIiwiZmFtaWx5IjoiUnVpeuKAkE1lcmNhZG8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiVsOtwq1jdG9yIE0uIiwiZmFtaWx5IjoiWmF2YWxhIn19XSwiaWRzIjp7Imlzc24iOlsiMDkyMS0zNDQ5IiwiMTg3OS0wNjU4Il0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDE2L2oucmVzY29ucmVjLjIwMjAuMTA1MTk5IiwibWFnIjoiMzA5NDYyNTEwNCIsInBtaWQiOiIzMzc0NjM2MSIsIm9wZW5hbGV4IjoiVzMwOTQ2MjUxMDQifSwidGl0bGUiOiJWYWx1aW5nIGVjb25vbWljIGltcGFjdCByZWR1Y3Rpb25zIG9mIG51dHJpZW50IHBvbGx1dGlvbiBmcm9tIGxpdmVzdG9jayB3YXN0ZSIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2oucmVzY29ucmVjLjIwMjAuMTA1MTk5IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5uY2JpLm5sbS5uaWguZ292L3BtYy9hcnRpY2xlcy83OTcwNTA1IiwiaXNzdWVkRGF0ZSI6IjIwMjAtMTAtMTUiLCJhY2Nlc3NlZERhdGUiOiIyMDI2LTAxIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJSZXNvdXJjZXMgQ29uc2VydmF0aW9uIGFuZCBSZWN5Y2xpbmciLCJ2b2x1bWUiOjE2NCwiaW1wYWN0RmFjdG9yIjoiMTEuMDEifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjoxMDUxOTksImxhc3RQYWdlIjoxMDUxOTl9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRWxzZXZpZXIgQlYifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjM5LCJjaXRlZEJ5Q291bnQiOjU3fX0sInF1b3RlIjoiU2VjdGlvbiBUaXRsZTogVmFsdWluZyBlY29ub21pYyBpbXBhY3QgcmVkdWN0aW9ucyBvZiBudXRyaWVudCBwb2xsdXRpb24gZnJvbSBsaXZlc3RvY2sgd2FzdGUgPiAxLiBJbnRyb2R1Y3Rpb25cbkNvbnRlbnQ6XG5JbiB0aGUgU3RhdGUgb2YgRmxvcmlkYSwgSEFCcyBoYXZlIHJlc3VsdGVkIGluIGEgbW9udGhseSBsb3NzIG9mIDIuOCBhbmQgMy43IG1pbGxpb24gVVNEIGNvcnJlc3BvbmRpbmcgdG8gcmVzdGF1cmFudCBhbmQgbG9kZ2luZyByZXZlbnVlLCByZXNwZWN0aXZlbHkgKCBbTGFya2luIGFuZCBBZGFtcywgMjAwN10oKSApLiBGcmVxdWVudCBvY2N1cnJlbmNlIG9mIEhBQnMgYWxzbyBsb3dlcnMgcHJvcGVydHkgdmFsdWVzIG9mIGxha2Vmcm9udCBwcm9wZXJ0aWVzLiBUaGUgbG9zcyBpbiBwcm9wZXJ0eSB2YWx1ZXMgYXJlIHRoZSBsYXJnZXN0IGltcGFjdCBiZWFyZXJzIG9mIGV1dHJvcGhpY2F0aW9uIHdpdGggYW4gZXN0aW1hdGVkIGF2ZXJhZ2UgZWNvbm9taWMgbG9zcyBvZiAxLjYgYmlsbGlvbiBVU0QgYW5udWFsbHkgKCBbRG9kZHMgZXQgYWwuLCAyMDA5XSgpICkuIn1d.

Transitioning to decentralized, adaptive governance is imperative, empowering local stakeholders with monitoring tools and incentives. While frameworks provide blueprints, their limitations highlight the need for hybrid models integrating citizen science and nature-based solutions, as explored in subsequent sections.

12. The Power of Citizen Science and Community-Led Monitoring

In the face of the "wicked" challenges posed by eutrophication and harmful algal blooms in Indian lakes, as highlighted by fragmented regulatory frameworks, citizen science emerges as a transformative, decentralized approach. Empowering local communities to monitor water quality, track algal proliferation, and report early warning signs fosters proactive prevention of severe blooms, complementing top-down policies with bottom-up resilience (Foulon et al., 2019)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IsOJdGllbm5lIiwiZmFtaWx5IjoiRm91bG9uIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFsYWluIE4uIiwiZmFtaWx5IjoiUm91c3NlYXUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiR2xlbm4iLCJmYW1pbHkiOiJCZW5veSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJSZWJlY2NhIEwuIiwiZmFtaWx5IjoiTm9ydGgifX1dLCJpZHMiOnsiaXNzbiI6WyIxMjAxLTMwODAiLCIyNDA4LTk0NDMiXSwiaXNibiI6W10sImRvaSI6IjEwLjIxNjYvd3Fyai4yMDE5LjAxMyIsIm1hZyI6IjI5NzM4NjAyODEiLCJvcGVuYWxleCI6IlcyOTczODYwMjgxIn0sInRpdGxlIjoiQSBnbG9iYWwgc2NhbiBvZiBob3cgdGhlIGlzc3VlIG9mIG51dHJpZW50IGxvYWRpbmcgYW5kIGhhcm1mdWwgYWxnYWwgYmxvb21zIGlzIGJlaW5nIGFkZHJlc3NlZCBieSBnb3Zlcm5tZW50cywgbm9uLWdvdmVybm1lbnRhbCBvcmdhbml6YXRpb25zLCBhbmQgdm9sdW50ZWVycyIsImFic3RyYWN0IjoiQWJzdHJhY3QgSGFybWZ1bCBhbGdhbCBibG9vbXMgKEhBQnMpIGluIGFxdWF0aWMgZWNvc3lzdGVtcyBhcmUgb2YgY29uY2VybiB3b3JsZHdpZGUuIFRoaXMgcmV2aWV3IGRlYWxzIHdpdGggaG93IGp1cmlzZGljdGlvbnMgYXJvdW5kIHRoZSB3b3JsZCBhcmUgYWRkcmVzc2luZyB0aGlzIHdhdGVyIHF1YWxpdHkgaXNzdWUgdG8gaW5mb3JtIHJlY29tbWVuZGF0aW9ucyByZWdhcmRpbmcgbnV0cmllbnQgbG9hZGluZyBhbmQgSEFCcyBpbiBNaXNzaXNxdW9pIEJheS1MYWtlIENoYW1wbGFpbiBhbmQgTGFrZSBNZW1waHJlbWFnb2c7IHRyYW5zYm91bmRhcnkgbGFrZXMgbG9jYXRlZCBpbiB0aGUgVVNBIGFuZCBDYW5hZGEgdGhhdCBzdWZmZXIgZnJvbSBzeW1wdG9tcyBvZiBldXRyb3BoaWNhdGlvbi4gQSBnbG9iYWwgc2NhbiBvZiB0aGUgbGl0ZXJhdHVyZSByZXN1bHRlZCBpbiB0aGUgY29uc2lkZXJhdGlvbiBvZiAxMiBjYXNlIHN0dWRpZXMgb2YgbGFyZ2Ugd2F0ZXIgYm9kaWVzIHdpdGhpbiBsYXJnZSB3YXRlcnNoZWRzLCBleGNsdWRpbmcgaW4tbGFrZSBnZW9lbmdpbmVlcmluZyBhcHByb2FjaGVzLiBBbHRob3VnaCBhbGwgb2YgdGhlIHN5c3RlbXMgZXhwZXJpZW5jZSBleGNlc3NpdmUgbnV0cmllbnQgbG9hZGluZywgdGhleSB2YXJ5IGluIHR3byBrZXkgd2F5czogc291cmNlcyBvZiBudXRyaWVudHMgYW5kIG1hbmlmZXN0YXRpb25zIG9mIGV1dHJvcGhpY2F0aW9uIHJhbmdpbmcgZnJvbSBIQUJzLCB0byBsaW1pdGVkIHJlY3JlYXRpb25hbCB1c2VzLCB0byB0aGUgYWRkaXRpb25hbCBjb21wbGV4aXR5IG9mIGludGVybmFsIGxvYWRpbmdzIGFuZCBmaXNoIGtpbGxzLCB1cCB0byBkcmlua2luZyB3YXRlciBzaHV0ZG93bnMuIFRoZSBjYXNlIHN0dWRpZXMgd2VyZSBhbmFseXplZCB3aXRoIHJlc3BlY3QgdG8gZm91ciBjYXRlZ29yaWVzIG9mIGFwcHJvYWNoZXMsIG5hbWVseTogKGkpIHJlZ3VsYXRvcnk7IChpaSkgaW5jZW50aXZlLWJhc2VkOyAoaWlpKSByaXNrIG1pdGlnYXRpb247IGFuZCAoaXYpIG91dHJlYWNoLCBlbmdhZ2VtZW50LCBhbmQgZWR1Y2F0aW9uYWwuIFdlIGZvdW5kIHRoYXQgdGhlIG1hbmFnZW1lbnQgZnJhbWV3b3JrcyBhcmUgYmFzZWQgb24gaW50ZWdyYXRlZCB3YXRlcnNoZWQgbWFuYWdlbWVudCBwbGFubmluZyBhbmQgbmF0aW9uYWwgc3RhbmRhcmRzLiBOYXRpb25hbCB3YXRlciBxdWFsaXR5IHN0YW5kYXJkcywgaG93ZXZlciwgYXJlIG5vdCBzdHJpbmdlbnQgZW5vdWdoIHRvIHByZXZlbnQgSEFCcy4gT3ZlcmFsbCwgaWRlbnRpZmllZCBjYXNlIHN0dWRpZXMgZGlkIG5vdCBzdWNjZXNzZnVsbHkgcmVtZWRpYXRlIEhBQnMsIHRoZXkgc2ltcGx5IG1hbmFnZWQgdGhlbS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjE2Ni93cXJqLjIwMTkuMDEzIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2l3YXBvbmxpbmUuY29tL3dxcmovYXJ0aWNsZS1wZGYvNTUvMS8xLzcwOTI2MS93cXJqYzA1NTAwMDEucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMTktMDktMTciLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA5IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJXYXRlciBRdWFsaXR5IFJlc2VhcmNoIEpvdXJuYWwiLCJpc3N1ZSI6MSwidm9sdW1lIjo1NSwiaW1wYWN0RmFjdG9yIjoiNy44MyJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjEsImxhc3RQYWdlIjoyM319LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJVV0EgUHVibGlzaGluZyJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6ODQsImNpdGVkQnlDb3VudCI6MjB9fSwicXVvdGUiOiJXZSBwcm9wb3NlIHNvbHV0aW9ucyBhbmQgcGF0aHMgZm9yd2FyZCB0byBhIHdpY2tlZCBwcm9ibGVtIHdoaWNoIHNob3VsZCBwcm92ZSBpbnNpZ2h0ZnVsIGZvciBwb2xpY3ktbWFrZXJzIGFuZCBtYW4tIGFnZXJzIGFsaWtlIHRocm91Z2ggdGhlIGZvbGxvd2luZyBmaW5kaW5ncy4gTmF0aW9uYWwgd2F0ZXIgcXVhbGl0eSBzdGFuZGFyZHMgaGF2ZSBub3QgcHJvdmVkIHN0cmluZ2VudCBlbm91Z2ggdG8gcHJlLSB2ZW50IEhBQnMuIElkZW50aWZpZWQgZnJhbWV3b3JrcyBhcmUgYWxsIGJhc2VkIG9uIGludGVncmF0ZWQgd2F0ZXJzaGVkIG1hbmFnZW1lbnQgYW5kIG5hdGlvbmFsIHN0YW5kYXJkcy4gTm8gYWN0dWFsIHN1Y2Nlc3Mgc3RvcmllcyByZWxhdGVkIHRvIHJlbWVkaWF0aW9uIG9mIEhBQnMgd2VyZSBpZGVudGlmaWVkOyB0aGV5IHdlcmUgc2ltcGx5IG1hbmFnZW1lbnQgc3Rvcmllcy4gVGhlIGVmZmljYWN5IG9mIE5QUyBCTVBzIGlzIHF1ZXN0aW9uZWQgdGhyb3VnaG91dCB0aGUgd29ybGQgZHVlIHRvIGluc3VmZmljaWVudCBldmlkZW5jZSB0byBkZXRlcm1pbmUgd2hldGhlciB0aGV5IGNhbiBtaXRpZ2F0ZSBIQUJzLiBNYXJrZXQtYmFzZWQgc3RyYXRlZ2llcyB0byBvZmZzZXQgcG9sbHV0aW9uIG9yIHByb21vdGUgdGFyZ2V0ZWQgQk1QIGFkb3B0aW9uIGFyZSBiZWNvbWluZyBwb3B1bGFyIGFuZCByZXN1bHRpbmcgaW4gY29zdC1lZmZlY3RpdmUgYW5kIGVmZmljaWVudCBhcHByb2FjaGVzLiBDb3N04oCTYmVuZWZpdCBhbmFseXNlcyBtYXkgaW5jcmVhc2UgdGhlIHVzZSBvZiB0ZWNobmljYWwgc29sLSB1dGlvbnMgb3ZlciBjb21wcmVoZW5zaXZlIGFwcHJvYWNoZXMuIEF0IHRoaXMgcG9pbnQsIHRoZXJlIGlzIG5vdCByZWFsbHkgYSB1bmlxdWUgY2FzZSBzdHVkeSB0aGF0IGNvdWxkIGJlIGFwcGxpZWQgdG8gTGFrZSBDaGFtcGxhaW4gYW5kIExha2UgTWVtcGhyZW1hZ29nLiBTcGVjaWZpYyBlbGVtZW50cyBvZiB0aGUgYXBwcm9hY2hlcyBpZGVudGlmaWVkIGluIHRoaXMgcmV2aWV3IGFuZCB1bmktIGZ5aW5nIHRoZW1lcywgaG93ZXZlciwgaG9sZCBwcm9taXNlcy4gVGhpcyBpcyB3aHkgd2UgYWR2b2NhdGUgZm9yIGNvbmR1Y3RpbmcgYSBzeXN0ZW1hdGljIGxpdGVyYXR1cmUgcmV2aWV3IHdpdGggcmVnYXJkIHRvIHRob3NlIHRoZW1lcyBpbiBvcmRlciB0byBkZXZlbG9wIHJlY29tbWVuLSBkYXRpb25zIHRhaWxvcmVkIHRvIHRoZSBMYWtlIE1lbXBocmVtYWdvZyBhbmQgTWlzc2lzcXVvaSBCYXktTGFrZSBDaGFtcGxhaW4gYXJlYXMuIEF0IGxhc3QsIHdlIHdvdWxkIGxpa2UgdG8gYWRkcmVzcyB0aGUgZWxlcGhhbnQtc2l6ZWQgcXVlcy0gdGlvbiBpbiB0aGUgcm9vbS4gV2lsbCBjbGltYXRlIGNoYW5nZSBpbmNyZWFzZSBIQUJzIGZyZXF1ZW5jeT8gSW50ZXJhY3RpdmUgZWZmZWN0cyBvZiBldXRyb3BoaWNhdGlvbiBhbmQgY2xpbWF0ZSBjaGFuZ2Ugb24gaGFybWZ1bCBjeWFub2JhY3RlcmlhbCBibG9vbXMgYXJlIGNvbXBsZXguICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxNSwiYmJveCI6eyJ0b3AiOjg1LjYwNTUxNDUyNjM2NzE5LCJsZWZ0Ijo0OS43OTQ4NzIyODM5MzU1NSwid2lkdGgiOjM5LjcxODY2MjI2MTk2Mjg5LCJoZWlnaHQiOjEuMzI4NjcxMzM2MTc0MDExMn19LHsicGFnZU51bWJlciI6MTUsImJib3giOnsidG9wIjo4Ny4zOTUzNjI4NTQwMDM5LCJsZWZ0Ijo0OS43OTQ4NzIyODM5MzU1NSwid2lkdGgiOjM5LjcwMjcwOTE5Nzk5ODA1LCJoZWlnaHQiOjEuMzI4NjcxMzM2MTc0MDExMn19LHsicGFnZU51bWJlciI6MTUsImJib3giOnsidG9wIjo4OS4xODUyMTExODE2NDA2MiwibGVmdCI6NDkuNzk0ODcyMjgzOTM1NTUsIndpZHRoIjozOS43MDIwOTUwMzE3MzgyOCwiaGVpZ2h0IjoxLjMyODY3MTMzNjE3NDAxMTJ9fSx7InBhZ2VOdW1iZXIiOjE1LCJiYm94Ijp7InRvcCI6OTAuOTc1MDU5NTA5Mjc3MzQsImxlZnQiOjQ5Ljc5NDg3MjI4MzkzNTU1LCJ3aWR0aCI6MzkuNzA0NjIwMzYxMzI4MTI1LCJoZWlnaHQiOjEuMzI4NjcxMzM2MTc0MDExMn19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjoxNC4wMjU5OTA0ODYxNDUwMiwibGVmdCI6MTAuNTAyMzIzMTUwNjM0NzY2LCJ3aWR0aCI6MzkuNzA1OTU1NTA1MzcxMDk0LCJoZWlnaHQiOjEuMzI4NjY5NDI4ODI1Mzc4NH19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjoxNS44MTU4NDE2NzQ4MDQ2ODgsImxlZnQiOjEwLjUwMjMyMzE1MDYzNDc2Niwid2lkdGgiOjM5LjcwMjI0NzYxOTYyODkwNiwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MTcuNjA1NjkxOTA5NzkwMDQsImxlZnQiOjEwLjUwMjE3NjI4NDc5MDAzOSwid2lkdGgiOjM5LjcwMDkyMzkxOTY3NzczNCwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MTkuMzk1NTQ0MDUyMTI0MDIzLCJsZWZ0IjoxMC41MDIxNzYyODQ3OTAwMzksIndpZHRoIjozOS43MDg5MDA0NTE2NjAxNTYsImhlaWdodCI6MS4zMjg2Njk0Mjg4MjUzNzg0fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjIxLjE4NTM5NDI4NzEwOTM3NSwibGVmdCI6MTAuNTAyMTc2Mjg0NzkwMDM5LCJ3aWR0aCI6MzkuNzEwMDgzMDA3ODEyNSwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MjIuOTc1MjQ0NTIyMDk0NzI3LCJsZWZ0IjoxMC41MDIxNzYyODQ3OTAwMzksIndpZHRoIjozOS43MDQwMjkwODMyNTE5NSwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MjQuNzY1MDk2NjY0NDI4NzEsImxlZnQiOjEwLjUwMjE3NjI4NDc5MDAzOSwid2lkdGgiOjM5LjcwMzg4MDMxMDA1ODU5NCwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MjYuNTU0OTQ4ODA2NzYyNjk1LCJsZWZ0IjoxMC41MDIwMjc1MTE1OTY2OCwid2lkdGgiOjM5LjcwMzg4MDMxMDA1ODU5NCwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MjguMzQ0Nzk5MDQxNzQ4MDQ3LCJsZWZ0IjoxMC41MDIwMjc1MTE1OTY2OCwid2lkdGgiOjM5LjcwMDAzODkwOTkxMjExLCJoZWlnaHQiOjEuMzI4NjY5NDI4ODI1Mzc4NH19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjozMC4xMzQ2NDczNjkzODQ3NjYsImxlZnQiOjEwLjUwMjAyNzUxMTU5NjY4LCJ3aWR0aCI6MzkuNzA0NjE2NTQ2NjMwODYsImhlaWdodCI6MS4zMjg2Njk0Mjg4MjUzNzg0fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjMxLjkyNDQ5NzYwNDM3MDExNywibGVmdCI6MTAuNTAxODc3Nzg0NzI5MDA0LCJ3aWR0aCI6MzkuNzA1Nzg3NjU4NjkxNDA2LCJoZWlnaHQiOjEuMzI4NjY5NDI4ODI1Mzc4NH19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjozMy43MTQzNTE2NTQwNTI3MzQsImxlZnQiOjEwLjUwMTcyOTAxMTUzNTY0NSwid2lkdGgiOjM5LjcxOTI0NTkxMDY0NDUzLCJoZWlnaHQiOjEuMzI4NjY3NDAyMjY3NDU2fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjM1LjUwNDIwMzc5NjM4NjcyLCJsZWZ0IjoxMC41MDE3MjkwMTE1MzU2NDUsIndpZHRoIjozOS43MDI3MDkxOTc5OTgwNSwiaGVpZ2h0IjoxLjMyODY2NzQwMjI2NzQ1Nn19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjozNy4yOTQwNTIxMjQwMjM0NCwibGVmdCI6MTAuNTAxNzI5MDExNTM1NjQ1LCJ3aWR0aCI6MzkuNzA1Nzk5MTAyNzgzMiwiaGVpZ2h0IjoxLjMyODY2NzQwMjI2NzQ1Nn19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjozOS4wODM5MDQyNjYzNTc0MiwibGVmdCI6MTAuNTAxNzI1MTk2ODM4Mzc5LCJ3aWR0aCI6MzkuNzA2NTM1MzM5MzU1NDcsImhlaWdodCI6MS4zMjg2Njc0MDIyNjc0NTZ9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6NDAuODczNzU2NDA4NjkxNDA2LCJsZWZ0IjoxMC41MDE3MjcxMDQxODcwMTIsIndpZHRoIjozOS43MjExNTMyNTkyNzczNDQsImhlaWdodCI6MS4zMjg2Njc0MDIyNjc0NTZ9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6NDIuNjYzNjA4NTUxMDI1MzksImxlZnQiOjEwLjUwMTcyNzEwNDE4NzAxMiwid2lkdGgiOjM5LjcwMzEzNjQ0NDA5MTgsImhlaWdodCI6MS4zMjg2Njc0MDIyNjc0NTZ9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6NDQuNDQ2MjA4OTUzODU3NDIsImxlZnQiOjEwLjUwMTcyNzEwNDE4NzAxMiwid2lkdGgiOjM5LjcwMDc3NTE0NjQ4NDM3NSwiaGVpZ2h0IjoxLjMyODY2NzQwMjI2NzQ1Nn19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjo0Ni4yMzYwNjEwOTYxOTE0MDYsImxlZnQiOjEwLjUwMTcyNzEwNDE4NzAxMiwid2lkdGgiOjM5LjcwNzcyNTUyNDkwMjM0NCwiaGVpZ2h0IjoxLjMyODY2NzQwMjI2NzQ1Nn19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjo0OC4wMjU5MTMyMzg1MjUzOSwibGVmdCI6MTAuNTAxNzI3MTA0MTg3MDEyLCJ3aWR0aCI6MjUuOTQxNjY3NTU2NzYyNjk1LCJoZWlnaHQiOjEuMzI4NjY3NDAyMjY3NDU2fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjQ5LjgxNTc2MTU2NjE2MjExLCJsZWZ0IjoxMy4xMjUwMTYyMTI0NjMzNzksIndpZHRoIjozNy4wODExODA1NzI1MDk3NjYsImhlaWdodCI6MS4zMjg2Njc0MDIyNjc0NTZ9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6NTEuNjA1NjEzNzA4NDk2MDk0LCJsZWZ0IjoxMC41MDE3MjcxMDQxODcwMTIsIndpZHRoIjozOS43MDkyMDU2Mjc0NDE0MDYsImhlaWdodCI6MS4zMjg2Njc0MDIyNjc0NTZ9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6NTMuMzk1NDYyMDM2MTMyODEsImxlZnQiOjEwLjUwMTcyNzEwNDE4NzAxMiwid2lkdGgiOjM5LjcxOTIyMzAyMjQ2MDk0LCJoZWlnaHQiOjEuMzI4NjY3NDAyMjY3NDU2fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjU1LjE4NTMxMDM2Mzc2OTUzLCJsZWZ0IjoxMC41MDE3MjcxMDQxODcwMTIsIndpZHRoIjozOS43MTA2NzgxMDA1ODU5NCwiaGVpZ2h0IjoxLjMyODY2NzQwMjI2NzQ1Nn19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjE2In19XQ%3D%3D. Unlike rigid national standards that often fail to capture site-specific dynamics, community-led initiatives leverage indigenous knowledge and accessible technologies to generate real-time data, bridging gaps in official monitoring (Khorasani & Zhu, 2021)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhhbWVkIiwiZmFtaWx5IjoiS2hvcmFzYW5pIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlpoZW5kdW8iLCJmYW1pbHkiOiJaaHUifX1dLCJpZHMiOnsiaXNzbiI6WyIwMDIyLTE2OTQiLCIxODc5LTI3MDciXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMTYvai5qaHlkcm9sLjIwMjEuMTI2ODg2IiwibWFnIjoiMzE3NzA3MTI5OCIsIm9wZW5hbGV4IjoiVzMxNzcwNzEyOTgifSwidGl0bGUiOiJQaG9zcGhvcnVzIHJldGVudGlvbiBpbiBsYWtlczogQSBjcml0aWNhbCByZWFzc2Vzc21lbnQgb2YgaHlwb3RoZXNlcyBhbmQgc3RhdGljIG1vZGVscyIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2ouamh5ZHJvbC4yMDIxLjEyNjg4NiIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9lYXJ0aGFyeGl2Lm9yZy9yZXBvc2l0b3J5L29iamVjdC8yNDgxL2Rvd25sb2FkLzUwNzQvIiwiaXNzdWVkRGF0ZSI6IjIwMjEtMDktMDIiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA4IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJKb3VybmFsIG9mIEh5ZHJvbG9neSIsInZvbHVtZSI6NjAzLCJpbXBhY3RGYWN0b3IiOiI3LjY0In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTI2ODg2LCJsYXN0UGFnZSI6MTI2ODg2fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkVsc2V2aWVyIEJWIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo3NywiY2l0ZWRCeUNvdW50IjoxMn19LCJxdW90ZSI6Ikhvd2V2ZXIsIGh1bWFuIGFjdGl2aXRpZXMgaGF2ZSA0OSBhY2NlbGVyYXRlZCB0aGUgZXV0cm9waGljYXRpb24gcHJvY2VzcyBieSBpbmNyZWFzaW5nIHRoZSBudXRyaWVudHMgZGVsaXZlcnkgdG8gdGhlIGFxdWF0aWMgc3lzdGVtcyA1MCAoTWVrb25uZW4gYW5kIEhvZWtzdHJhLCAyMDE4KS4gVGh1cywgYW50aHJvcG9nZW5pYyBldXRyb3BoaWNhdGlvbiBpcyBvbmUgb2YgdGhlIG1vc3QgaW1wb3J0YW50IDUxIGVsZW1lbnRzIG9mIGZyZXNoIGFuZCBtYXJpbmUgd2F0ZXIgcXVhbGl0eSBkZXRlcmlvcmF0aW9uIChIdSBldCBhbC4sIDIwMTk7IFNtaXRoIGFuZCBTY2hpbmRsZXIsIDUyIDIwMDkpLiBPbmUgZGlyZWN0IGNvbnNlcXVlbmNlIG9mIGFudGhyb3BvZ2VuaWMgZXV0cm9waGljYXRpb24gY29tZXMgaW4gdGhlIGZvcm0gb2YgbWFzc2l2ZSBhbGdhbCA1MyBibG9vbXMgKEdyYW7DqWxpIGV0IGFsLiwgMjAwODsgSGVpc2xlciBldCBhbC4sIDIwMDgpLCB3aGljaCBhcmUgcHJlZGljdGVkIHRvIGJlIGludGVuc2lmaWVkIHVuZGVyIDU0IHdhcm1lciB3YXRlciB0ZW1wZXJhdHVyZXMgYXMgY2xpbWF0ZSBjaGFuZ2VzIChHb2JsZXIsIDIwMjA7IE11a3VuZGFuIGV0IGFsLiwgMjAyMCkuIDU1IEV1dHJvcGhpY2F0aW9uIGlzIGEg4oCcd2lja2Vk4oCdIHByb2JsZW0sIHdoaWNoIGlzIHRoZSBjb25zZXF1ZW5jZSBvZiB2YXJpb3VzIHByb2Nlc3NlcyB0aGF0IG9wZXJhdGUgNTYgY3VtdWxhdGl2ZWx5LiBDb25zaWRlcmluZyB0aGUgdW5pcXVlbmVzcyBvZiBlYWNoIGxha2UgYW5kIGl0cyBzdXJyb3VuZGluZyBhcmVhLCB0aGVyZSBpcyBubyBicm9hZGx5IDU3IGFwcGxpY2FibGUgc2V0IG9mIGJlc3QgbWFuYWdlbWVudCBwcmFjdGljZXMgdGhhdCBjYW4gYmUgYXBwbGllZCBpbiB3YXRlcnNoZWRzIHRvIG1pdGlnYXRlIDU4IHBob3NwaG9ydXMgbG9hZGluZyBhbmQgaXRzIGltcGFjdCBvbiBhbGwgbGFrZXMgKFRob3JudG9uIGV0IGFsLiwgMjAxMykuIEhlbmNlLCBldXRyb3BoaWNhdGlvbiA1OSBtYW5hZ2VtZW50IGFuZCBsYWtlIHJlc3RvcmF0aW9uIG5lZWQgaW50ZWdyYXRlZCBwbGFucyB0aGF0IGFyZSBub3Qgb25seSBzY2llbnRpZmljYWxseSB2YWxpZCBidXQgNjAgYWxzbyBzb2Npby1lY29ub21pY2FsbHkgc2F0aXNmeWluZyAoR2lic29uIGV0IGFsLiwgMjAwMCkuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MzYuMTY4NDM0MTQzMDY2NDA2LCJsZWZ0IjoxMS43Njg2MjgxMjA0MjIzNjMsIndpZHRoIjo3Ni45NjMzOTQxNjUwMzkwNiwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjM2LjcxODQzMzM4MDEyNjk1LCJsZWZ0Ijo2Ljk4MDM5MjQ1NjA1NDY4NzUsIndpZHRoIjoyLjI1MDk1NzQ4OTAxMzY3MiwiaGVpZ2h0IjoxLjM5Mzk0MDQ0ODc2MDk4NjN9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjozOS42NTMyODU5ODAyMjQ2MSwibGVmdCI6Ni45ODAzOTI0NTYwNTQ2ODc1LCJ3aWR0aCI6ODEuNzYxNDI4ODMzMDA3ODEsImhlaWdodCI6Mi4wMjEyMTE2MjQxNDU1MDh9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo0My4xMzgxMzQwMDI2ODU1NSwibGVmdCI6Ni45ODAzOTI0NTYwNTQ2ODc1LCJ3aWR0aCI6ODEuNzU1NTYxODI4NjEzMjgsImhlaWdodCI6Mi4wMjEyMTE2MjQxNDU1MDh9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo0Ni42MjI5ODIwMjUxNDY0ODQsImxlZnQiOjYuOTgwMzkyNDU2MDU0Njg3NSwid2lkdGgiOjgxLjc0MjgwNTQ4MDk1NzAzLCJoZWlnaHQiOjIuMDIxMjExNjI0MTQ1NTA4fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6NTAuMTA3ODMwMDQ3NjA3NDIsImxlZnQiOjYuOTgwMzkyNDU2MDU0Njg3NSwid2lkdGgiOjgxLjc1MjYxNjg4MjMyNDIyLCJoZWlnaHQiOjIuMDIxMjExNjI0MTQ1NTA4fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6NTMuNTk1MjAzMzk5NjU4MiwibGVmdCI6Ni45ODAzOTI0NTYwNTQ2ODc1LCJ3aWR0aCI6ODEuNzQ4NzAzMDAyOTI5NjksImhlaWdodCI6Mi4wMjEyMTE2MjQxNDU1MDh9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo1Ny4wODAwNDc2MDc0MjE4NzUsImxlZnQiOjYuOTgwMzkyNDU2MDU0Njg3NSwid2lkdGgiOjczLjMxMDQ1NTMyMjI2NTYyLCJoZWlnaHQiOjIuMDIxMjExNjI0MTQ1NTA4fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6NjEuNTY0OTAzMjU5Mjc3MzQ0LCJsZWZ0Ijo2Ljk4MDM5MjQ1NjA1NDY4NzUsIndpZHRoIjo4MS43NTc1MTQ5NTM2MTMyOCwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjY1LjA0OTc0MzY1MjM0Mzc1LCJsZWZ0Ijo2Ljk4MDM5MjQ1NjA1NDY4NzUsIndpZHRoIjo4MS43NTk0NzU3MDgwMDc4MSwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjY4LjUzNDU5MTY3NDgwNDY5LCJsZWZ0Ijo2Ljk4MDM5MjQ1NjA1NDY4NzUsIndpZHRoIjo4MS43MDM5MzM3MTU4MjAzMSwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjcyLjAxOTQ1NDk1NjA1NDY5LCJsZWZ0Ijo2Ljk4MDM5MjQ1NjA1NDY4NzUsIndpZHRoIjo4MS43NTA2NTYxMjc5Mjk2OSwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjc1LjUwODA4NzE1ODIwMzEyLCJsZWZ0Ijo2Ljk4MDM5MjQ1NjA1NDY4NzUsIndpZHRoIjo4MS43NTc1MTQ5NTM2MTMyOCwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjc4Ljk5MjkzNTE4MDY2NDA2LCJsZWZ0IjoxMS43Njg2MjgxMjA0MjIzNjMsIndpZHRoIjo3Ni45NzMxOTc5MzcwMTE3MiwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEyNjg4OCJ9fV0%3D. For instance, volunteers can deploy low-cost sensors for parameters like dissolved oxygen, pH, turbidity, and nutrient levels critical indicators of bloom onset, as hypoxia (DO < 2 mg/L) precedes cyanobacterial dominance (Jansen et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkpvYWNoaW0iLCJmYW1pbHkiOiJKYW5zZW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiR2F2aW4gTC4iLCJmYW1pbHkiOiJTaW1wc29uIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikdlc2EgQS4iLCJmYW1pbHkiOiJXZXloZW5tZXllciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJMYXVyYSBILiIsImZhbWlseSI6IkjDpHJrw7ZuZW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW5kcmV3IE0uIiwiZmFtaWx5IjoiUGF0ZXJzb24ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUGF1bCBBLiBkZWwiLCJmYW1pbHkiOiJHaW9yZ2lvIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ill2ZXMgVC4iLCJmYW1pbHkiOiJQcmFpcmllIn19XSwiaWRzIjp7Imlzc24iOlsiMTc1OC02NzhYIiwiMTc1OC02Nzk4Il0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDM4L3M0MTU1OC0wMjQtMDIwNTgtMyIsIm9wZW5hbGV4IjoiVzQ0MDAyMDgyMDAiLCJleGEiOiJodHRwczovL3d3dy5uYXR1cmUuY29tL2FydGljbGVzL3M0MTU1OC0wMjQtMDIwNTgtMz9lcnJvcj1jb29raWVzX25vdF9zdXBwb3J0ZWQmY29kZT1hNzRhMjVmZi1hNThkLTRkZjgtOTM2Zi0yNjJlMmUzZTkxOTgifSwidGl0bGUiOiJDbGltYXRlLWRyaXZlbiBkZW94eWdlbmF0aW9uIG9mIG5vcnRoZXJuIGxha2VzIiwiYWJzdHJhY3QiOiJBYnN0cmFjdCBPeHlnZW4gZGVwbGV0aW9uIGNvbnN0aXR1dGVzIGEgbWFqb3IgdGhyZWF0IHRvIGxha2UgZWNvc3lzdGVtcyBhbmQgdGhlIHNlcnZpY2VzIHRoZXkgcHJvdmlkZS4gTW9zdCBvZiB0aGUgd29ybGTigJlzIGxha2VzIGFyZSBsb2NhdGVkICZndDs0NcKwIE4sIHdoZXJlIGFjY2VsZXJhdGVkIGNsaW1hdGUgd2FybWluZyBhbmQgZWxldmF0ZWQgY2FyYm9uIGxvYWRzIG1pZ2h0IHNldmVyZWx5IGluY3JlYXNlIHRoZSByaXNrIG9mIGh5cG94aWEsIGJ1dCB0aGlzIGhhcyBub3QgYmVlbiBzeXN0ZW1hdGljYWxseSBleGFtaW5lZC4gSGVyZSBhbmFseXNpcyBvZiAyLjYgbWlsbGlvbiB3YXRlciBxdWFsaXR5IG9ic2VydmF0aW9ucyBmcm9tIDgsMjg4IGxha2VzIHNob3dzIHRoYXQgYmV0d2VlbiAxOTYwIGFuZCAyMDIyLCBtb3N0IG5vcnRoZXJuIGxha2VzIGV4cGVyaWVuY2VkIHJhcGlkIGRlb3h5Z2VuYXRpb24gc3Ryb25nbHkgbGlua2VkIHRvIGNsaW1hdGUtZHJpdmVuIHByb2xvbmdhdGlvbiBvZiBzdW1tZXIgc3RyYXRpZmljYXRpb24uIE94eWdlbiBsZXZlbHMgZGV0ZXJpb3JhdGVkIG1vc3QgaW4gc21hbGwgbGFrZXMgKCZsdDsxMCBoYSkgb3dpbmcgdG8gdGhlaXIgZ3JlYXRlciB2b2x1bWV0cmljIG94eWdlbiBkZW1hbmQgYW5kIHN1cmZhY2Ugd2FybWluZyByYXRlcywgd2hpbGUgdGhlIGxhcmdlc3QgbGFrZXMgZ2FpbmVkIG94eWdlbiB1bmRlciBtaW5pbWFsIHN0cmF0aWZpY2F0aW9uIGNoYW5nZXMgYW5kIGltcHJvdmVkIGFlcmF0aW9uIGF0IHNwcmluZyBvdmVydHVybnMuIFNlYXNvbmFsIG94eWdlbiBjb25zdW1wdGlvbiByYXRlcyBkZWNsaW5lZCwgZGVzcGl0ZSB3aWRlc3ByZWFkIGJyb3duaW5nLiBQcm9saWZlcmF0aW5nIGFub3hpYSBlbmhhbmNlZCBzZWFzb25hbCBpbnRlcm5hbCBsb2FkaW5nIG9mIEMsIFAgYW5kIE4gYnV0IGRlcGxldGVkIFAgbG9uZy10ZXJtLCBpbmRpY2F0aW5nIHRoYXQgZGVveHlnZW5hdGlvbiBjYW4gZXhoYXVzdCByZWRveC1zZW5zaXRpdmUgZnJhY3Rpb25zIG9mIHNlZGltZW50IG51dHJpZW50IHJlc2Vydm9pcnMuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjEwMzgvczQxNTU4LTAyNC0wMjA1OC0zIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAzOC9zNDE1NTgtMDI0LTAyMDU4LTMiLCJpc3N1ZWREYXRlIjoiMjAyNC0wNy0wMSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTEiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6Ik5hdHVyZSBDbGltYXRlIENoYW5nZSIsImlzc3VlIjo4LCJ2b2x1bWUiOjE0LCJpbXBhY3RGYWN0b3IiOiIxNS4wMiJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjgzMiwibGFzdFBhZ2UiOjgzOH19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJOYXR1cmUgUG9ydGZvbGlvIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo4MCwiY2l0ZWRCeUNvdW50IjozN319LCJxdW90ZSI6IkFlcm9iaWMgcmVzcGlyYXRpb24gY2FuIGRlcGxldGUgb3h5Z2VuIGNvbmNlbnRyYXRpb25zIGF0IGRlcHRoIHdoZW4gc2Vhc29uYWwgc3RyYXRpZmljYXRpb24gcHJldmVudHMgYWVyYXRpb24gb2YgdGhlIHdhdGVyIGNvbHVtbls1XShodHRwczovL3d3dy5uYXR1cmUuY29tL2FydGljbGVzL3M0MTU1OC0wMjQtMDIwNTgtMyNyZWYtQ1I1KS4gVGhpcyBkZXRyaW1lbnRhbCBjb25kaXRpb24sIHRlcm1lZCBoeXBveGlhIChET%2BKAiTzigIky4oCJbWfigIls4oiSMSksIGRpbWluaXNoZXMgdGhlIGNvbGQtd2F0ZXIgaGFiaXRhdCBvZiBhcXVhdGljIGxpZmUsIGluY2x1ZGluZyBjb21tZXJjaWFsbHkgaW1wb3J0YW50IGZpc2hbNl0oaHR0cHM6Ly93d3cubmF0dXJlLmNvbS9hcnRpY2xlcy9zNDE1NTgtMDI0LTAyMDU4LTMjcmVmLUNSNikgYW5kIGNhbiBjYXVzZSBtYXNzIG1vcnRhbGl0eSBpZiBwZXJzaXN0ZW50WzddKGh0dHBzOi8vd3d3Lm5hdHVyZS5jb20vYXJ0aWNsZXMvczQxNTU4LTAyNC0wMjA1OC0zI3JlZi1DUjcpLiBBbm94aWEgKERP4oCJPOKAiTAuNeKAiW1n4oCJbOKIkjEpIHByb21vdGVzIHRoZSByZWR1Y3RpdmUgZGlzc29sdXRpb24gb2YgbWluZXJhbC1ib3VuZCBjYXJib24gYW5kIHBob3NwaG9ydXNbOF0oaHR0cHM6Ly93d3cubmF0dXJlLmNvbS9hcnRpY2xlcy9zNDE1NTgtMDI0LTAyMDU4LTMjcmVmLUNSOCksIFs5XShodHRwczovL3d3dy5uYXR1cmUuY29tL2FydGljbGVzL3M0MTU1OC0wMjQtMDIwNTgtMyNyZWYtQ1I5KSB3aGljaCBmdWVscyBoYXJtZnVsIGFsZ2FsIGJsb29tc1sxMF0oaHR0cHM6Ly93d3cubmF0dXJlLmNvbS9hcnRpY2xlcy9zNDE1NTgtMDI0LTAyMDU4LTMjcmVmLUNSMTApIGFuZCBsaW1pdHMgbWljcm9iaWFsIG94aWRhdGlvbiBvZiBtZXRoYW5lLCBhIHBvdGVudCBncmVlbmhvdXNlIGdhc1sxMV0oaHR0cHM6Ly93d3cubmF0dXJlLmNvbS9hcnRpY2xlcy9zNDE1NTgtMDI0LTAyMDU4LTMjcmVmLUNSMTEpLiJ9XQ%3D%3D. This participatory model not only enhances data granularity but also cultivates stewardship, addressing governance lapses such as underfunded monitoring in lakes like Bellandur and Dal (Paerl et al., 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhhbnMgVy4iLCJmYW1pbHkiOiJQYWVybCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLLkUuIiwiZmFtaWx5IjoiSGF2ZW5zIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik4uUy4iLCJmYW1pbHkiOiJIYWxsIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlQuRy4iLCJmYW1pbHkiOiJPdHRlbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNLiIsImZhbWlseSI6IlpodSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJIYWlodWkiLCJmYW1pbHkiOiJYdSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJHLiIsImZhbWlseSI6IlpodSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJCLiIsImZhbWlseSI6IlFpbiJ9fV0sImlkcyI6eyJpc3NuIjpbXSwiaXNibiI6W10sImRvaSI6IjEwLjE3NjE1LzRua2Qtems5MyIsIm9wZW5hbGV4IjoiVzQzMDcwNjI0MjkifSwidGl0bGUiOiJNaXRpZ2F0aW5nIGEgZ2xvYmFsIGV4cGFuc2lvbiBvZiB0b3hpYyBjeWFub2JhY3RlcmlhbCBibG9vbXM6IENvbmZvdW5kaW5nIGVmZmVjdHMgYW5kIGNoYWxsZW5nZXMgcG9zZWQgYnkgY2xpbWF0ZSBjaGFuZ2UiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTc2MTUvNG5rZC16azkzIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2Nkci5saWIudW5jLmVkdS9kb3dubG9hZHMvMDU3NDIyODM0IiwiaXNzdWVkRGF0ZSI6IjIwMjAtMDEtMDEiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTAxIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJDYXJvbGluYSBEaWdpdGFsIFJlcG9zaXRvcnkgKFVuaXZlcnNpdHkgb2YgTm9ydGggQ2Fyb2xpbmEgYXQgQ2hhcGVsIEhpbGwpIn0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlVuaXZlcnNpdHkgb2YgTm9ydGggQ2Fyb2xpbmEgYXQgQ2hhcGVsIEhpbGwifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjAsImNpdGVkQnlDb3VudCI6MH19LCJxdW90ZSI6IlRoYXQgcmVsYXRpb25zaGlwIGhhcyBzZXJ2ZWQgYXMgdGhlIGZvdW5kYXRpb24gZm9yIHNldHRpbmcgIG51dHJpZW50IGxvYWRpbmcgdGFyZ2V0cyBpbiBsYWtlcyBhcm91bmQgdGhlIHdvcmxkLCBhbmQgY2FyZWZ1bGx5ICBkZXNpZ25lZCBleHBlcmltZW50cyBtYXkgZWx1Y2lkYXRlIHRvIHdoYXQgZXh0ZW50IHRlbXBlcmF0dXJlICBpbmNyZWFzZSBhZmZlY3RzIHRoZSB0aHJlc2hvbGQgYXQgd2hpY2ggdGhlIGZyZXF1ZW5jeSBvZiBvY2N1ciAgcmVuY2Ugb2YgQ3lhbm9IQUJzIHF1aWNrbHkgcmlzZXMgd2l0aCBpbmNyZWFzaW5nIE4gYW5kIFAuIENvbmNsdXNpb25zIFJlY2VudCBzdHVkaWVzIGluZGljYXRlIHRoYXQgdGhlIHN5bmVyZ2lzdGljIGNvbWJpbmF0aW9uIG9mIGFudGhyb3BvZ2VuaWMgbnV0cmllbnQgbG9hZGluZywgcmlzaW5nIHRlbXBlcmF0dXJlcywgZW5oYW5jZWQgIHZlcnRpY2FsIHN0cmF0aWZpY2F0aW9uLCBpbmNyZWFzZWQgcmVzaWRlbmNlIHRpbWUgYW5kIG1vcmUgIGV4dHJlbWUgY2xpbWF0aWMgY29uZGl0aW9ucyBvdmVyYWxsIHdpbGwgZmF2b3VyIGN5YW5vYmFjdGVyaWFsICBkb21pbmFuY2UgaW4gYSB3aWRlIHJhbmdlIG9mIGFxdWF0aWMgZWNvc3lzdGVtcy4gVGhlIGV4cGFuICBzaW9uIG9mIEN5YW5vSEFCcyBoYXMgc2VyaW91cyBjb25zZXF1ZW5jZXMgZm9yIGh1bWFuIHdhdGVyICBzdXBwbGllcywgZmlzaGVyaWVzIGFuZCByZWNyZWF0aW9uYWwgcmVzb3VyY2VzLiBFY29zeXN0ZW0gc2NhbGUgIHBoeXNpY2FsIGNoZW1pY2FsIGJpb3RpYyBtaXRpZ2F0aW9uIHN0cmF0ZWdpZXMgd2lsbCBoYXZlIHRvICBpbmNvcnBvcmF0ZSBudXRyaWVudCAobW9zdCBvZnRlbiBib3RoIE4gYW5kIFApIGlucHV0IHJlZHVjdGlvbnMuICBUaGUgbWFnbml0dWRlcyBvZiB0aGVzZSByZWR1Y3Rpb25zIHdpbGwgbmVlZCB0byBiZSBzeXN0ZW0gIHNwZWNpZmljIGFuZCB3aWxsIGxpa2VseSBuZWVkIHRvIGJlIGFkanVzdGVkIHdpdGggY2hhbmdpbmcgY2xpICBtYXRpYyBjb25kaXRpb25zLiBTcGVjaWZpY2FsbHksIHdhdGVyIHF1YWxpdHkgbWFuYWdlcnMgd2lsbCBoYXZlICB0byBhY2NvbW1vZGF0ZSB3YXJtaW5nIGFuZCBncmVhdGVyIGh5ZHJvbG9naWNhbCB2YXJpYWJpbGl0eSAgYW5kIGV4dHJlbWVuZXNzIHJlc3VsdGluZyBmcm9tIGNsaW1hdGUgY2hhbmdlIGluIGZvcm11bGF0aW5nICB0aGVpciBzdHJhdGVnaWVzLiBBIGtleSBsb25nIHRlcm0gY29udHJvbCB3ZSBjYW4gZXhlcnQgdG8gcmVkdWNlICB0aGUgcmF0ZSBhbmQgZXh0ZW50IG9mIGdsb2JhbCB3YXJtaW5nIGlzIGN1cmJpbmcgZ3JlZW5ob3VzZSBnYXMgIGVtaXNzaW9ucy4gV2l0aG91dCB0aGlzIGVzc2VudGlhbCBzdGVwLCBmdXR1cmUgd2FybWluZyB0cmVuZHMsICBoeWRyb2xvZ2ljYWwgZXh0cmVtZXMgKHJlY29yZCByYWluZmFsbHMgYW5kIGRyb3VnaHRzKSBhbmQgdGhlaXIgIGVmZmVjdHMgb24gYXF1YXRpYyBlY29zeXN0ZW1zIHdpbGwgY29udGludWUgdG8gcHJvbW90ZSBvcHBvciAgdHVuaXN0aWMgQ3lhbm9IQUJzLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjoxNC4xNDkyMzM4MTgwNTQyLCJsZWZ0IjoxMy4xMjg1OTUzNTIxNzI4NTIsIndpZHRoIjozNy4wODIxNzIzOTM3OTg4MywiaGVpZ2h0IjoxLjUxNjYwMjI3Nzc1NTczNzN9fSx7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6MTUuNDUxNTA4NTIyMDMzNjkxLCJsZWZ0IjoxMy4xMjg1NDQ4MDc0MzQwODIsIndpZHRoIjozNy4wODQ4MzEyMzc3OTI5NywiaGVpZ2h0IjoxLjUxNjYwMzIzMTQzMDA1Mzd9fSx7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6MTYuNzUzNzg0MTc5Njg3NSwibGVmdCI6MTMuMTI4NTQyOTAwMDg1NDUsIndpZHRoIjozNy4wODMyMDk5OTE0NTUwOCwiaGVpZ2h0IjoxLjUxNjYwNDE4NTEwNDM3MDF9fSx7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6MTguMDU2MDUzMTYxNjIxMDk0LCJsZWZ0IjoxMy4xMjg1NDQ4MDc0MzQwODIsIndpZHRoIjozNi42MTUwNzQxNTc3MTQ4NDQsImhlaWdodCI6MS41MTY2MDQxODUxMDQzNzAxfX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjE5LjQ2ODM3ODA2NzAxNjYsImxlZnQiOjEzLjEyODU0NDgwNzQzNDA4Miwid2lkdGgiOjM0LjI5MDUzMTE1ODQ0NzI2NiwiaGVpZ2h0IjoxLjI0NTAxMTIxMDQ0MTU4OTR9fSx7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6MjIuMjI2MTQyODgzMzAwNzgsImxlZnQiOjEzLjEyODU1MDUyOTQ3OTk4LCJ3aWR0aCI6Ny42NDk4NzM3MzM1MjA1MDgsImhlaWdodCI6MS4xMzU0ODMxNDU3MTM4MDYyfX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjIzLjczNDg0OTkyOTgwOTU3LCJsZWZ0IjoxMy4xMjg1MzUyNzA2OTA5MTgsIndpZHRoIjozNy4wODkyODY4MDQxOTkyMiwiaGVpZ2h0IjoyNi4wODg2NDAyMTMwMTI2OTV9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIxMSJ9fV0%3D.

Practical implementation involves equipping communities with user-friendly tools: smartphone apps for photographic algal identification via AI, citizen kits for on-site testing of total phosphorus (often exceeding 100 μg/L in affected lakes), and community dashboards for anomaly alerts(Foulon et al., 2019)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IsOJdGllbm5lIiwiZmFtaWx5IjoiRm91bG9uIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFsYWluIE4uIiwiZmFtaWx5IjoiUm91c3NlYXUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiR2xlbm4iLCJmYW1pbHkiOiJCZW5veSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJSZWJlY2NhIEwuIiwiZmFtaWx5IjoiTm9ydGgifX1dLCJpZHMiOnsiaXNzbiI6WyIxMjAxLTMwODAiLCIyNDA4LTk0NDMiXSwiaXNibiI6W10sImRvaSI6IjEwLjIxNjYvd3Fyai4yMDE5LjAxMyIsIm1hZyI6IjI5NzM4NjAyODEiLCJvcGVuYWxleCI6IlcyOTczODYwMjgxIn0sInRpdGxlIjoiQSBnbG9iYWwgc2NhbiBvZiBob3cgdGhlIGlzc3VlIG9mIG51dHJpZW50IGxvYWRpbmcgYW5kIGhhcm1mdWwgYWxnYWwgYmxvb21zIGlzIGJlaW5nIGFkZHJlc3NlZCBieSBnb3Zlcm5tZW50cywgbm9uLWdvdmVybm1lbnRhbCBvcmdhbml6YXRpb25zLCBhbmQgdm9sdW50ZWVycyIsImFic3RyYWN0IjoiQWJzdHJhY3QgSGFybWZ1bCBhbGdhbCBibG9vbXMgKEhBQnMpIGluIGFxdWF0aWMgZWNvc3lzdGVtcyBhcmUgb2YgY29uY2VybiB3b3JsZHdpZGUuIFRoaXMgcmV2aWV3IGRlYWxzIHdpdGggaG93IGp1cmlzZGljdGlvbnMgYXJvdW5kIHRoZSB3b3JsZCBhcmUgYWRkcmVzc2luZyB0aGlzIHdhdGVyIHF1YWxpdHkgaXNzdWUgdG8gaW5mb3JtIHJlY29tbWVuZGF0aW9ucyByZWdhcmRpbmcgbnV0cmllbnQgbG9hZGluZyBhbmQgSEFCcyBpbiBNaXNzaXNxdW9pIEJheS1MYWtlIENoYW1wbGFpbiBhbmQgTGFrZSBNZW1waHJlbWFnb2c7IHRyYW5zYm91bmRhcnkgbGFrZXMgbG9jYXRlZCBpbiB0aGUgVVNBIGFuZCBDYW5hZGEgdGhhdCBzdWZmZXIgZnJvbSBzeW1wdG9tcyBvZiBldXRyb3BoaWNhdGlvbi4gQSBnbG9iYWwgc2NhbiBvZiB0aGUgbGl0ZXJhdHVyZSByZXN1bHRlZCBpbiB0aGUgY29uc2lkZXJhdGlvbiBvZiAxMiBjYXNlIHN0dWRpZXMgb2YgbGFyZ2Ugd2F0ZXIgYm9kaWVzIHdpdGhpbiBsYXJnZSB3YXRlcnNoZWRzLCBleGNsdWRpbmcgaW4tbGFrZSBnZW9lbmdpbmVlcmluZyBhcHByb2FjaGVzLiBBbHRob3VnaCBhbGwgb2YgdGhlIHN5c3RlbXMgZXhwZXJpZW5jZSBleGNlc3NpdmUgbnV0cmllbnQgbG9hZGluZywgdGhleSB2YXJ5IGluIHR3byBrZXkgd2F5czogc291cmNlcyBvZiBudXRyaWVudHMgYW5kIG1hbmlmZXN0YXRpb25zIG9mIGV1dHJvcGhpY2F0aW9uIHJhbmdpbmcgZnJvbSBIQUJzLCB0byBsaW1pdGVkIHJlY3JlYXRpb25hbCB1c2VzLCB0byB0aGUgYWRkaXRpb25hbCBjb21wbGV4aXR5IG9mIGludGVybmFsIGxvYWRpbmdzIGFuZCBmaXNoIGtpbGxzLCB1cCB0byBkcmlua2luZyB3YXRlciBzaHV0ZG93bnMuIFRoZSBjYXNlIHN0dWRpZXMgd2VyZSBhbmFseXplZCB3aXRoIHJlc3BlY3QgdG8gZm91ciBjYXRlZ29yaWVzIG9mIGFwcHJvYWNoZXMsIG5hbWVseTogKGkpIHJlZ3VsYXRvcnk7IChpaSkgaW5jZW50aXZlLWJhc2VkOyAoaWlpKSByaXNrIG1pdGlnYXRpb247IGFuZCAoaXYpIG91dHJlYWNoLCBlbmdhZ2VtZW50LCBhbmQgZWR1Y2F0aW9uYWwuIFdlIGZvdW5kIHRoYXQgdGhlIG1hbmFnZW1lbnQgZnJhbWV3b3JrcyBhcmUgYmFzZWQgb24gaW50ZWdyYXRlZCB3YXRlcnNoZWQgbWFuYWdlbWVudCBwbGFubmluZyBhbmQgbmF0aW9uYWwgc3RhbmRhcmRzLiBOYXRpb25hbCB3YXRlciBxdWFsaXR5IHN0YW5kYXJkcywgaG93ZXZlciwgYXJlIG5vdCBzdHJpbmdlbnQgZW5vdWdoIHRvIHByZXZlbnQgSEFCcy4gT3ZlcmFsbCwgaWRlbnRpZmllZCBjYXNlIHN0dWRpZXMgZGlkIG5vdCBzdWNjZXNzZnVsbHkgcmVtZWRpYXRlIEhBQnMsIHRoZXkgc2ltcGx5IG1hbmFnZWQgdGhlbS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjE2Ni93cXJqLjIwMTkuMDEzIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2l3YXBvbmxpbmUuY29tL3dxcmovYXJ0aWNsZS1wZGYvNTUvMS8xLzcwOTI2MS93cXJqYzA1NTAwMDEucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMTktMDktMTciLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA5IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJXYXRlciBRdWFsaXR5IFJlc2VhcmNoIEpvdXJuYWwiLCJpc3N1ZSI6MSwidm9sdW1lIjo1NSwiaW1wYWN0RmFjdG9yIjoiNy44MyJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjEsImxhc3RQYWdlIjoyM319LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJVV0EgUHVibGlzaGluZyJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6ODQsImNpdGVkQnlDb3VudCI6MjB9fSwicXVvdGUiOiJXZSBwcm9wb3NlIHNvbHV0aW9ucyBhbmQgcGF0aHMgZm9yd2FyZCB0byBhIHdpY2tlZCBwcm9ibGVtIHdoaWNoIHNob3VsZCBwcm92ZSBpbnNpZ2h0ZnVsIGZvciBwb2xpY3ktbWFrZXJzIGFuZCBtYW4tIGFnZXJzIGFsaWtlIHRocm91Z2ggdGhlIGZvbGxvd2luZyBmaW5kaW5ncy4gTmF0aW9uYWwgd2F0ZXIgcXVhbGl0eSBzdGFuZGFyZHMgaGF2ZSBub3QgcHJvdmVkIHN0cmluZ2VudCBlbm91Z2ggdG8gcHJlLSB2ZW50IEhBQnMuIElkZW50aWZpZWQgZnJhbWV3b3JrcyBhcmUgYWxsIGJhc2VkIG9uIGludGVncmF0ZWQgd2F0ZXJzaGVkIG1hbmFnZW1lbnQgYW5kIG5hdGlvbmFsIHN0YW5kYXJkcy4gTm8gYWN0dWFsIHN1Y2Nlc3Mgc3RvcmllcyByZWxhdGVkIHRvIHJlbWVkaWF0aW9uIG9mIEhBQnMgd2VyZSBpZGVudGlmaWVkOyB0aGV5IHdlcmUgc2ltcGx5IG1hbmFnZW1lbnQgc3Rvcmllcy4gVGhlIGVmZmljYWN5IG9mIE5QUyBCTVBzIGlzIHF1ZXN0aW9uZWQgdGhyb3VnaG91dCB0aGUgd29ybGQgZHVlIHRvIGluc3VmZmljaWVudCBldmlkZW5jZSB0byBkZXRlcm1pbmUgd2hldGhlciB0aGV5IGNhbiBtaXRpZ2F0ZSBIQUJzLiBNYXJrZXQtYmFzZWQgc3RyYXRlZ2llcyB0byBvZmZzZXQgcG9sbHV0aW9uIG9yIHByb21vdGUgdGFyZ2V0ZWQgQk1QIGFkb3B0aW9uIGFyZSBiZWNvbWluZyBwb3B1bGFyIGFuZCByZXN1bHRpbmcgaW4gY29zdC1lZmZlY3RpdmUgYW5kIGVmZmljaWVudCBhcHByb2FjaGVzLiBDb3N04oCTYmVuZWZpdCBhbmFseXNlcyBtYXkgaW5jcmVhc2UgdGhlIHVzZSBvZiB0ZWNobmljYWwgc29sLSB1dGlvbnMgb3ZlciBjb21wcmVoZW5zaXZlIGFwcHJvYWNoZXMuIEF0IHRoaXMgcG9pbnQsIHRoZXJlIGlzIG5vdCByZWFsbHkgYSB1bmlxdWUgY2FzZSBzdHVkeSB0aGF0IGNvdWxkIGJlIGFwcGxpZWQgdG8gTGFrZSBDaGFtcGxhaW4gYW5kIExha2UgTWVtcGhyZW1hZ29nLiBTcGVjaWZpYyBlbGVtZW50cyBvZiB0aGUgYXBwcm9hY2hlcyBpZGVudGlmaWVkIGluIHRoaXMgcmV2aWV3IGFuZCB1bmktIGZ5aW5nIHRoZW1lcywgaG93ZXZlciwgaG9sZCBwcm9taXNlcy4gVGhpcyBpcyB3aHkgd2UgYWR2b2NhdGUgZm9yIGNvbmR1Y3RpbmcgYSBzeXN0ZW1hdGljIGxpdGVyYXR1cmUgcmV2aWV3IHdpdGggcmVnYXJkIHRvIHRob3NlIHRoZW1lcyBpbiBvcmRlciB0byBkZXZlbG9wIHJlY29tbWVuLSBkYXRpb25zIHRhaWxvcmVkIHRvIHRoZSBMYWtlIE1lbXBocmVtYWdvZyBhbmQgTWlzc2lzcXVvaSBCYXktTGFrZSBDaGFtcGxhaW4gYXJlYXMuIEF0IGxhc3QsIHdlIHdvdWxkIGxpa2UgdG8gYWRkcmVzcyB0aGUgZWxlcGhhbnQtc2l6ZWQgcXVlcy0gdGlvbiBpbiB0aGUgcm9vbS4gV2lsbCBjbGltYXRlIGNoYW5nZSBpbmNyZWFzZSBIQUJzIGZyZXF1ZW5jeT8gSW50ZXJhY3RpdmUgZWZmZWN0cyBvZiBldXRyb3BoaWNhdGlvbiBhbmQgY2xpbWF0ZSBjaGFuZ2Ugb24gaGFybWZ1bCBjeWFub2JhY3RlcmlhbCBibG9vbXMgYXJlIGNvbXBsZXguICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxNSwiYmJveCI6eyJ0b3AiOjg1LjYwNTUxNDUyNjM2NzE5LCJsZWZ0Ijo0OS43OTQ4NzIyODM5MzU1NSwid2lkdGgiOjM5LjcxODY2MjI2MTk2Mjg5LCJoZWlnaHQiOjEuMzI4NjcxMzM2MTc0MDExMn19LHsicGFnZU51bWJlciI6MTUsImJib3giOnsidG9wIjo4Ny4zOTUzNjI4NTQwMDM5LCJsZWZ0Ijo0OS43OTQ4NzIyODM5MzU1NSwid2lkdGgiOjM5LjcwMjcwOTE5Nzk5ODA1LCJoZWlnaHQiOjEuMzI4NjcxMzM2MTc0MDExMn19LHsicGFnZU51bWJlciI6MTUsImJib3giOnsidG9wIjo4OS4xODUyMTExODE2NDA2MiwibGVmdCI6NDkuNzk0ODcyMjgzOTM1NTUsIndpZHRoIjozOS43MDIwOTUwMzE3MzgyOCwiaGVpZ2h0IjoxLjMyODY3MTMzNjE3NDAxMTJ9fSx7InBhZ2VOdW1iZXIiOjE1LCJiYm94Ijp7InRvcCI6OTAuOTc1MDU5NTA5Mjc3MzQsImxlZnQiOjQ5Ljc5NDg3MjI4MzkzNTU1LCJ3aWR0aCI6MzkuNzA0NjIwMzYxMzI4MTI1LCJoZWlnaHQiOjEuMzI4NjcxMzM2MTc0MDExMn19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjoxNC4wMjU5OTA0ODYxNDUwMiwibGVmdCI6MTAuNTAyMzIzMTUwNjM0NzY2LCJ3aWR0aCI6MzkuNzA1OTU1NTA1MzcxMDk0LCJoZWlnaHQiOjEuMzI4NjY5NDI4ODI1Mzc4NH19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjoxNS44MTU4NDE2NzQ4MDQ2ODgsImxlZnQiOjEwLjUwMjMyMzE1MDYzNDc2Niwid2lkdGgiOjM5LjcwMjI0NzYxOTYyODkwNiwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MTcuNjA1NjkxOTA5NzkwMDQsImxlZnQiOjEwLjUwMjE3NjI4NDc5MDAzOSwid2lkdGgiOjM5LjcwMDkyMzkxOTY3NzczNCwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MTkuMzk1NTQ0MDUyMTI0MDIzLCJsZWZ0IjoxMC41MDIxNzYyODQ3OTAwMzksIndpZHRoIjozOS43MDg5MDA0NTE2NjAxNTYsImhlaWdodCI6MS4zMjg2Njk0Mjg4MjUzNzg0fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjIxLjE4NTM5NDI4NzEwOTM3NSwibGVmdCI6MTAuNTAyMTc2Mjg0NzkwMDM5LCJ3aWR0aCI6MzkuNzEwMDgzMDA3ODEyNSwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MjIuOTc1MjQ0NTIyMDk0NzI3LCJsZWZ0IjoxMC41MDIxNzYyODQ3OTAwMzksIndpZHRoIjozOS43MDQwMjkwODMyNTE5NSwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MjQuNzY1MDk2NjY0NDI4NzEsImxlZnQiOjEwLjUwMjE3NjI4NDc5MDAzOSwid2lkdGgiOjM5LjcwMzg4MDMxMDA1ODU5NCwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MjYuNTU0OTQ4ODA2NzYyNjk1LCJsZWZ0IjoxMC41MDIwMjc1MTE1OTY2OCwid2lkdGgiOjM5LjcwMzg4MDMxMDA1ODU5NCwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MjguMzQ0Nzk5MDQxNzQ4MDQ3LCJsZWZ0IjoxMC41MDIwMjc1MTE1OTY2OCwid2lkdGgiOjM5LjcwMDAzODkwOTkxMjExLCJoZWlnaHQiOjEuMzI4NjY5NDI4ODI1Mzc4NH19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjozMC4xMzQ2NDczNjkzODQ3NjYsImxlZnQiOjEwLjUwMjAyNzUxMTU5NjY4LCJ3aWR0aCI6MzkuNzA0NjE2NTQ2NjMwODYsImhlaWdodCI6MS4zMjg2Njk0Mjg4MjUzNzg0fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjMxLjkyNDQ5NzYwNDM3MDExNywibGVmdCI6MTAuNTAxODc3Nzg0NzI5MDA0LCJ3aWR0aCI6MzkuNzA1Nzg3NjU4NjkxNDA2LCJoZWlnaHQiOjEuMzI4NjY5NDI4ODI1Mzc4NH19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjozMy43MTQzNTE2NTQwNTI3MzQsImxlZnQiOjEwLjUwMTcyOTAxMTUzNTY0NSwid2lkdGgiOjM5LjcxOTI0NTkxMDY0NDUzLCJoZWlnaHQiOjEuMzI4NjY3NDAyMjY3NDU2fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjM1LjUwNDIwMzc5NjM4NjcyLCJsZWZ0IjoxMC41MDE3MjkwMTE1MzU2NDUsIndpZHRoIjozOS43MDI3MDkxOTc5OTgwNSwiaGVpZ2h0IjoxLjMyODY2NzQwMjI2NzQ1Nn19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjozNy4yOTQwNTIxMjQwMjM0NCwibGVmdCI6MTAuNTAxNzI5MDExNTM1NjQ1LCJ3aWR0aCI6MzkuNzA1Nzk5MTAyNzgzMiwiaGVpZ2h0IjoxLjMyODY2NzQwMjI2NzQ1Nn19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjozOS4wODM5MDQyNjYzNTc0MiwibGVmdCI6MTAuNTAxNzI1MTk2ODM4Mzc5LCJ3aWR0aCI6MzkuNzA2NTM1MzM5MzU1NDcsImhlaWdodCI6MS4zMjg2Njc0MDIyNjc0NTZ9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6NDAuODczNzU2NDA4NjkxNDA2LCJsZWZ0IjoxMC41MDE3MjcxMDQxODcwMTIsIndpZHRoIjozOS43MjExNTMyNTkyNzczNDQsImhlaWdodCI6MS4zMjg2Njc0MDIyNjc0NTZ9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6NDIuNjYzNjA4NTUxMDI1MzksImxlZnQiOjEwLjUwMTcyNzEwNDE4NzAxMiwid2lkdGgiOjM5LjcwMzEzNjQ0NDA5MTgsImhlaWdodCI6MS4zMjg2Njc0MDIyNjc0NTZ9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6NDQuNDQ2MjA4OTUzODU3NDIsImxlZnQiOjEwLjUwMTcyNzEwNDE4NzAxMiwid2lkdGgiOjM5LjcwMDc3NTE0NjQ4NDM3NSwiaGVpZ2h0IjoxLjMyODY2NzQwMjI2NzQ1Nn19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjo0Ni4yMzYwNjEwOTYxOTE0MDYsImxlZnQiOjEwLjUwMTcyNzEwNDE4NzAxMiwid2lkdGgiOjM5LjcwNzcyNTUyNDkwMjM0NCwiaGVpZ2h0IjoxLjMyODY2NzQwMjI2NzQ1Nn19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjo0OC4wMjU5MTMyMzg1MjUzOSwibGVmdCI6MTAuNTAxNzI3MTA0MTg3MDEyLCJ3aWR0aCI6MjUuOTQxNjY3NTU2NzYyNjk1LCJoZWlnaHQiOjEuMzI4NjY3NDAyMjY3NDU2fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjQ5LjgxNTc2MTU2NjE2MjExLCJsZWZ0IjoxMy4xMjUwMTYyMTI0NjMzNzksIndpZHRoIjozNy4wODExODA1NzI1MDk3NjYsImhlaWdodCI6MS4zMjg2Njc0MDIyNjc0NTZ9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6NTEuNjA1NjEzNzA4NDk2MDk0LCJsZWZ0IjoxMC41MDE3MjcxMDQxODcwMTIsIndpZHRoIjozOS43MDkyMDU2Mjc0NDE0MDYsImhlaWdodCI6MS4zMjg2Njc0MDIyNjc0NTZ9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6NTMuMzk1NDYyMDM2MTMyODEsImxlZnQiOjEwLjUwMTcyNzEwNDE4NzAxMiwid2lkdGgiOjM5LjcxOTIyMzAyMjQ2MDk0LCJoZWlnaHQiOjEuMzI4NjY3NDAyMjY3NDU2fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjU1LjE4NTMxMDM2Mzc2OTUzLCJsZWZ0IjoxMC41MDE3MjcxMDQxODcwMTIsIndpZHRoIjozOS43MTA2NzgxMDA1ODU5NCwiaGVpZ2h0IjoxLjMyODY2NzQwMjI2NzQ1Nn19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjE2In19XQ%3D%3D. In Chilika Lagoon, Odisha, local fishers have adapted hydrological monitoring post-inlet interventions, tracking salinity shifts and nutrient spikes from aquaculture that sustain Microcystis blooms (Jilbert et al., 2020; Nair & Nayak, 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRvbSIsImZhbWlseSI6IkppbGJlcnQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUmFvdWzigJBNYXJpZSIsImZhbWlseSI6IkNvdXR1cmUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQnJpYW4iLCJmYW1pbHkiOiJIdXNlciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLYWxldmkiLCJmYW1pbHkiOiJTYWxvbmVuIn19XSwiaWRzIjp7Imlzc24iOlsiMDAxOC04MTU4IiwiMTU3My01MTE3Il0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDA3L3MxMDc1MC0wMjAtMDQ0NTcteCIsIm1hZyI6IjMxMDA0NTkwNTAiLCJvcGVuYWxleCI6IlczMTAwNDU5MDUwIn0sInRpdGxlIjoiUHJlZmFjZTogUmVzdG9yYXRpb24gb2YgZXV0cm9waGljIGxha2VzOiBjdXJyZW50IHByYWN0aWNlcyBhbmQgZnV0dXJlIGNoYWxsZW5nZXMiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAwNy9zMTA3NTAtMDIwLTA0NDU3LXgiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vbGluay5zcHJpbmdlci5jb20vY29udGVudC9wZGYvMTAuMTAwNy9zMTA3NTAtMDIwLTA0NDU3LXgucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjAtMTEtMTYiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA3IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJIeWRyb2Jpb2xvZ2lhIiwiaXNzdWUiOjIxLCJ2b2x1bWUiOjg0NywiaW1wYWN0RmFjdG9yIjoiMy4wMCJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjQzNDMsImxhc3RQYWdlIjo0MzU3fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlNwcmluZ2VyIFNjaWVuY2UrQnVzaW5lc3MgTWVkaWEifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjk1LCJjaXRlZEJ5Q291bnQiOjU4fX0sInF1b3RlIjoiU2ltaWxhciBkZWdyYWRhdGlvbiBvZiBmcmVzaHdhdGVyIGVjb3N5c3RlbXMgaXMgb2JzZXJ2ZWQgd29ybGR3aWRlLCB3aXRoIHRoZSB0aW1lbGluZSBvZiBodW1hbiBwcmVzc3VyZSBkaWN0YXRlZCBieSB0cmVuZHMgaW4gdXJiYW5pemF0aW9uIGFuZCBpbnRlbnNpdmUgYWdyaWN1bHR1cmUgaW4gZGlmZmVyZW50IGdlb2dyYXBoaWMgcmVnaW9ucyAoZS5nLiwgVmVubmVtbyBldCBhbC4sIDIwMDkpLiBUaGUgcHJpbmNpcGFsIGRyaXZlciBvZiBlY29zeXN0ZW0gc3RyZXNzIGluIG1vc3QgYXF1YXRpYyBzeXN0ZW1zIGlzIGV1dHJvcGhpY2F0aW9uLCB0aGUgZXhjZXNzIGdyb3d0aCBvZiBhbGdhZSBkcml2ZW4gYnkgYW50aHJvcG9nZW5pYyBpbnB1dHMgb2YgdGhlIG1hY3JvbnV0cmllbnRzIG5pdHJvZ2VuIChOKSBhbmQgcGhvc3Bob3J1cyAoUCkgKFNtaXRoLCAyMDAzOyBCaXJrIGV0IGFsLiwgMjAyMCkuIE51dHJpZW50cyBhcmUgZW1pdHRlZCBmcm9tIGJvdGggcG9pbnQgYW5kIGRpZmZ1c2Ugc291cmNlcyBpbiB0aGUgaHVtYW4gZW52aXJvbm1lbnQgYW5kIGVudGVyIGFxdWF0aWMgc3lzdGVtcyB2aWEgc3VyZmFjZSBydW5vZmYsIGFzIHdlbGwgYXMgZ3JvdW5kLXdhdGVyIGFuZCBhdG1vc3BoZXJpYyBkZXBvc2l0aW9uIChQYWVybCwgMTk5NykuIEV1dHJvcGhpY2F0aW9uIG9mdGVuIGxlYWRzIHRvIHRoZSBkb21pbmFuY2Ugb2YgcHJpbWFyeSBwcm9kdWN0aW9uIGJ5IGN5YW5vYmFjdGVyaWEsIGluY2x1ZGluZyB0b3hpYyBnZW5lcmEgc3VjaCBhcyBEb2xpY2hvc3Blcm11bSwgQXBoYW5pem9tZW5vbiwgYW5kIE1pY3JvY3lzdGlzIChIdWlzbWFuIGV0IGFsLiwgMjAxOCksIGFzIHdlbGwgYXMgb3h5Z2VuIGRlcGxldGlvbiBhbmQgaGFiaXRhdCBkZWdyYWRhdGlvbiBpbiBsYWtlcyAoQXJlbmQgZXQgYWwuLCAyMDExKS4gQmVjYXVzZSBsYWtlcyBwcm92aWRlIGEgcmFuZ2Ugb2YgY3J1Y2lhbCBlY29zeXN0ZW0gc2VydmljZXMgKFdpbHNvbiAmIENhcnBlbnRlciwgMTk5OSksIHRoZXJlIGlzIGEgZ3Jvd2luZyBuZWVkIGZvciBtYW5hZ2VtZW50IGFwcHJvYWNoZXMgdGhhdCByZXN0b3JlIHdhdGVyIHF1YWxpdHkgaW4gZnJlc2h3YXRlciBzeXN0ZW1zLiBXaXRoaW4gdGhlIGZyYW1ld29yayBvZiBVbml0ZWQgTmF0aW9ucyBTdXN0YWluYWJsZSBEZXZlbG9wLW1lbnQgR29hbCA2IChDbGVhbiBXYXRlciBhbmQgU2FuaXRhdGlvbiksIHNldmVyYWwgZGlzdGluY3QgdGFyZ2V0cyByZWxhdGUgdG8gdGhlIGltcHJvdmVtZW50IG9mIHdhdGVyIHF1YWxpdHkgYW5kIHByb3RlY3Rpb24gb2YgYXF1YXRpYyBlY29zeXN0ZW1zIGJ5IDIwMzAgKFVuaXRlZCBOYXRpb25zLCAyMDE1KS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyOC4yOTg4NjQzNjQ2MjQwMjMsImxlZnQiOjguNTQ5MjA1NzgwMDI5Mjk3LCJ3aWR0aCI6MzkuMzcxNDk0MjkzMjEyODksImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MjkuOTkxMTM0NjQzNTU0Njg4LCJsZWZ0Ijo4LjU0OTIwNTc4MDAyOTI5Nywid2lkdGgiOjM5LjM2NTExOTkzNDA4MjAzLCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMxLjY4MzQwNjgyOTgzMzk4NCwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNjk2NjMyMzg1MjUzOSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozMy4zNjc5NzMzMjc2MzY3MiwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNzU2NzkwMTYxMTMyOCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNS4wNjAyNDkzMjg2MTMyOCwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNzA5NjQwNTAyOTI5NywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNi43NTI1MTc3MDAxOTUzMSwibGVmdCI6OC41NDkyMDg2NDEwNTIyNDYsIndpZHRoIjozOS4zNzY5NjA3NTQzOTQ1MywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC40MzcwODgwMTI2OTUzMSwibGVmdCI6OC41NDkyMDg2NDEwNTIyNDYsIndpZHRoIjozOS4zNjY1Njk1MTkwNDI5NywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0MC4xMjkzNTYzODQyNzczNDQsImxlZnQiOjguNTQ5MjA4NjQxMDUyMjQ2LCJ3aWR0aCI6MzkuMzY0Mzc5ODgyODEyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0MS44MjE2MjQ3NTU4NTkzNzUsImxlZnQiOjguNTQ5MjA4NjQxMDUyMjQ2LCJ3aWR0aCI6MzkuMzY1NjczMDY1MTg1NTUsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDMuNTA2MTk1MDY4MzU5Mzc1LCJsZWZ0Ijo4LjU0OTM4OTgzOTE3MjM2Mywid2lkdGgiOjM5LjM3MDM5MTg0NTcwMzEyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0NS4xOTg0NjM0Mzk5NDE0MDYsImxlZnQiOjguNTQ5NTcyOTQ0NjQxMTEzLCJ3aWR0aCI6MzkuMzY1ODQ0NzI2NTYyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ni44OTA3MzU2MjYyMjA3LCJsZWZ0Ijo4LjU0OTU3Mjk0NDY0MTExMywid2lkdGgiOjM5LjM3Mjk1NTMyMjI2NTYyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0OC41NzUzMDIxMjQwMjM0NCwibGVmdCI6OC41NDk1NzI5NDQ2NDExMTMsIndpZHRoIjozOS4zNzI0MjEyNjQ2NDg0NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1MC4yNjc1NzgxMjUsImxlZnQiOjguNTQ5NTc2NzU5MzM4Mzc5LCJ3aWR0aCI6MzkuMzcxMTI0MjY3NTc4MTI1LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjUxLjk1OTg0MjY4MTg4NDc2NiwibGVmdCI6OC41NDk1NzY3NTkzMzgzNzksIndpZHRoIjozOS4zNjc4MzU5OTg1MzUxNTYsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTMuNTc0MTE5NTY3ODcxMDk0LCJsZWZ0Ijo4LjU0OTU3Njc1OTMzODM3OSwid2lkdGgiOjM5LjM3MDc3NzEzMDEyNjk1LCJoZWlnaHQiOjEuMzM2ODk1NTg1MDYwMTE5Nn19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjU1LjI2NjM5MTc1NDE1MDM5LCJsZWZ0Ijo4LjU0OTc2MDgxODQ4MTQ0NSwid2lkdGgiOjM5LjM2NjU4MDk2MzEzNDc2NiwiaGVpZ2h0IjoxLjMzNjg5NTU4NTA2MDExOTZ9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1Ny4wMjg5NTM1NTIyNDYwOTQsImxlZnQiOjguNTQ5OTQ0ODc3NjI0NTEyLCJ3aWR0aCI6MzkuMzYyMDI2MjE0NTk5NjEsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTguNzEzNTIzODY0NzQ2MDk0LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjkuMjk5OTgyMDcwOTIyODUyLCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjYwLjQwNTc5MjIzNjMyODEyNSwibGVmdCI6MTAuNjIyNDU1NTk2OTIzODI4LCJ3aWR0aCI6MzcuMjg3ODY4NDk5NzU1ODYsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NjIuMDk4MDYwNjA3OTEwMTU2LCJsZWZ0Ijo4LjU0OTk0MzkyMzk1MDE5NSwid2lkdGgiOjM5LjM3MzMyNTM0NzkwMDM5LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjYzLjc4MjYzMDkyMDQxMDE1NiwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzMxMjY5ODM2NDI1OCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2NS40NzQ5MDY5MjEzODY3MiwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNjUxMTk5MzQwODIwMywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2Ny4xNjcxNzUyOTI5Njg3NSwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNjk0ODAxMzMwNTY2NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2OC43ODE0NDgzNjQyNTc4MSwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzczMzg0MDk0MjM4MywiaGVpZ2h0IjoxLjMzNjg5MTUzMTk0NDI3NX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjcwLjU0NDAxMzk3NzA1MDc4LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjM5LjM2OTMwODQ3MTY3OTY5LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjcyLjIzNjI4OTk3ODAyNzM0LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjM5LjM3MjAzMjE2NTUyNzM0NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo3My45MjA4NDUwMzE3MzgyOCwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzEzMDczNzMwNDY4NzUsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNDM0NCJ9fSx7Im9yaWdpbiI6NCwid29yayI6eyJ0eXBlIjo4LCJzdWJ0eXBlIjp7Im9uZW9mS2luZCI6ImFydGljbGVUeXBlIiwiYXJ0aWNsZVR5cGUiOjF9LCJjb250cmlidXRvcnMiOlt7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJOYXZ5YSBWaWtyYW1hbiIsImZhbWlseSI6Ik5haXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUHJhdGVlcCBLdW1hciIsImZhbWlseSI6Ik5heWFrIn19XSwiaWRzIjp7Imlzc24iOlsiMjI5Ni03NzQ1Il0sImlzYm4iOltdLCJkb2kiOiIxMC4zMzg5L2ZtYXJzLjIwMjMuMTA4NzI5NiIsIm9wZW5hbGV4IjoiVzQzODI3MjMxMzMifSwidGl0bGUiOiJVbmNvdmVyaW5nIHdhdGVyIHF1YWxpdHkgYW5kIGV2YWx1YXRpbmcgdnVsbmVyYWJpbGl0aWVzIG9mIHNtYWxsLXNjYWxlIGZpc2hlcmllcyBpbiBDaGlsaWthIExhZ29vbiwgSW5kaWEiLCJhYnN0cmFjdCI6IlNtYWxsLXNjYWxlIGZpc2hlcmllcyAoU1NGcykgcGxheSBhIHN0cm9uZyByb2xlIGluIHN1c3RhaW5pbmcgbWlsbGlvbnMgb2YgbGl2ZWxpaG9vZHMsIGZvb2Qgc2VjdXJpdHksIG51dHJpdGlvbiwgYW5kIGluY29tZSBnbG9iYWxseSBidXQgdGhlIGZpc2hlcnMgZW5nYWdlZCBpbiB0aGlzIHNlY3RvciBzaW11bHRhbmVvdXNseSBleHBlcmllbmNlIGhpZ2ggbGV2ZWxzIG9mIHZ1bG5lcmFiaWxpdHkgYW5kIHByb2Nlc3NlcyBvZiBtYXJnaW5hbGlzYXRpb24uIFNldmVyYWwgZmFjdG9ycyBhcmUgYXR0cmlidXRlZCB0byB0aGUgbXVsdGlkaW1lbnNpb25hbCB2dWxuZXJhYmlsaXRpZXMgdGhlIHNtYWxsLXNjYWxlIGZpc2hlcnMgZXhwZXJpZW5jZSwgc3Bhbm5pbmcgYm90aCBuYXR1cmFsIChlLmcuLCBuYXR1cmFsIGRpc2FzdGVycywgZWNvc3lzdGVtIGNoYW5nZSkgYW5kIGFudGhyb3BvZ2VuaWMgKGUuZy4sIHBvbGljeSBjaGFuZ2UsIGh5ZHJvbG9naWNhbCBpbnRlcnZlbnRpb25zLCBhcXVhY3VsdHVyZSkgcHJlc3N1cmVzLiBXaGlsZSB0aGVyZSBpcyBtdWNoIGxpdGVyYXR1cmUgb24gdmFyaW91cyBuYXR1cmFsIGFuZCBodW1hbiBkcml2ZXJzIG9mIHZ1bG5lcmFiaWxpdHkgaW4gc21hbGwtc2NhbGUgZmlzaGVyeSBjb21tdW5pdGllcywgYW4gYWJzZW5jZSBvZiByZXNlYXJjaCBjb25uZWN0aW5nIHZ1bG5lcmFiaWxpdHkgd2l0aCB3YXRlciBxdWFsaXR5IGlzIGV2aWRlbnQuIEZpc2hlciBjb21tdW5pdGllcyBvZnRlbiB0YWxrIGFib3V0IGZpc2ggaW4gcmVsYXRpb24gdG8gdGhlIGhlYWx0aCBvZiB0aGVpciBhcXVhdGljIGhhYml0YXRzIHdoZXJlaW4gd2F0ZXIgcXVhbGl0eSBpcyBzZWVuIGFzIGEga2V5IHBhcmFtZXRlci4gVGhlIGxpbmsgYmV0d2VlbiBoZWFsdGh5IGZpc2ggYW5kIGdvb2QgcXVhbGl0eSB3YXRlciBoYXMgc2lnbmlmaWNhbnQgaW1wbGljYXRpb25zIGZvciBzdHJvbmcgYW5kIHZpYWJsZSBmaXNoaW5nIGNvbW11bml0aWVzLiBUaGlzIHBhcGVyIGV4YW1pbmVzIHRoZXNlIGxpbmtzIGZ1cnRoZXIgYnkgZm9jdXNpbmcgb24gdGhlIG5hdHVyZSBvZiB2dWxuZXJhYmlsaXRpZXMgY2F1c2VkIGJ5IHdhdGVyIHF1YWxpdHkgY2hhbmdlcyBpbiB0aGUgc21hbGwtc2NhbGUgZmlzaGVyeSBzeXN0ZW0gb2YgQ2hpbGlrYSBMYWdvb24gaW4gSW5kaWEuIFdlIHVuZGVydGFrZSBkZXRhaWxlZCBhbmFseXNpcyBvZiB0aGUgaW52YXNpdmUgc2hyaW1wIGFxdWFjdWx0dXJlIGFjdGl2aXRpZXMgYW5kIGh5ZHJvbG9naWNhbCBpbnRlcnZlbnRpb25zIGZvciBvcGVuaW5nIG9mIGEgbGFnb29uIGlubGV0IHdpdGggdGhlIEJheSBvZiBCZW5nYWwgYXMgdHdvIGRvbWluYW50IGRyaXZlcnMgYWR2ZXJzZWx5IGltcGFjdGluZyB3YXRlciBxdWFsaXR5IGFuZCBpbmNyZWFzaW5nIHZ1bG5lcmFiaWxpdGllcyBvZiB0aGUgZW50aXJlIHNtYWxsLXNjYWxlIGZpc2hlcmllcyBzb2NpYWwtZWNvbG9naWNhbCBzeXN0ZW0uIE91ciBhbmFseXNpcyBzdWdnZXN0cyB0aGF0IHRoZXJlIGFyZSBzdHJvbmcgaW50ZXJjb25uZWN0aW9ucyBiZXR3ZWVuIGNoYW5nZXMgaW4gd2F0ZXIgcXVhbGl0eSBhbmQgdGhlIGxldmVscyBvZiB2dWxuZXJhYmlsaXRpZXMgaW4gdGhlIFNTRnMgb2YgQ2hpbGlrYSBMYWdvb24uIFBvbGx1dGFudHMgc3VjaCBhcyBwZXN0aWNpZGVzLCBhbmQgb3JnYW5pYyBjb21wb3VuZHMgYWNjdW11bGF0ZSBpbiBmaXNoIHRpc3N1ZXMgYW5kIGFmZmVjdCB0aGVpciBncm93dGgsIHJlcHJvZHVjdGlvbiwgYW5kIG92ZXJhbGwgaGVhbHRoLiBUaGlzIGxlZCB0byBkZWNsaW5lcyBpbiBmaXNoIHBvcHVsYXRpb25zLCBtYWtpbmcgaXQgbW9yZSBkaWZmaWN1bHQgZm9yIGZpc2hlcnMgdG8gbWFrZSBhIGxpdmluZy4gSW4gYWRkaXRpb24gdG8gZGlyZWN0IGltcGFjdHMgb24gZmlzaCBwb3B1bGF0aW9ucywgcG9vciB3YXRlciBxdWFsaXR5IGFsc28gaGFzIGluZGlyZWN0IGVmZmVjdHMgb24gdGhlIHNvY2lhbCBhbmQgZWNvbm9taWMgdnVsbmVyYWJpbGl0eSBvZiBTU0ZzLiBGb3IgZXhhbXBsZSwgY29udGFtaW5hdGlvbiBvZiB3YXRlciBzb3VyY2VzIGxlZCB0byB0aGUgcmVkdWNlZCBudW1iZXIgb2YgZmlzaCBzcGVjaWVzIHJlZHVjaW5nIHRoZSBhbW91bnQgb2YgdGltZSBmaXNoZXJzIGNhbiBzcGVuZCBvbiBmaXNoaW5nIGFjdGl2aXRpZXMuIFRoaXMgYWxzbyBhZmZlY3RlZCB0aGUgbWFya2V0YWJpbGl0eSBvZiBmaXNoIHByb2R1Y3RzLCByZWR1Y2VkIGluY29tZSBhbmQgaW5jcmVhc2VkIHBvdmVydHkuIFRvIGZ1bGx5IHVuZGVyc3RhbmQgdGhlIGludGVyY29ubmVjdGlvbnMgYmV0d2VlbiB3YXRlciBxdWFsaXR5IGFuZCB2dWxuZXJhYmlsaXR5IGluIFNTRnMgaW4gQ2hpbGlrYSBMYWdvb24sIGl0IGlzIGltcG9ydGFudCB0byBjb25zaWRlciBib3RoIGVudmlyb25tZW50YWwgYW5kIHNvY2lhbCBmYWN0b3JzLCBhcyB3ZWxsIGFzIHRoZSBjb21wbGV4IGZlZWRiYWNrIGxvb3BzIGJldHdlZW4gdGhlc2UgZmFjdG9ycy4gVGhlIHN0dWR5IGhlbHBzIGluIGJyaWRnaW5nIGEgY3J1Y2lhbCBnYXAgaW4gb3VyIHVuZGVyc3RhbmRpbmcgb2YgdGhlIHJvbGUgb2Ygd2F0ZXIgcXVhbGl0eSBpbiB2dWxuZXJhYmlsaXR5IGFuYWx5c2lzIHdpdGhpbiByZXNvdXJjZSBkZXBlbmRlbnQgY29tbXVuaXRpZXMuIFdlIGNvbmNsdWRlIHdpdGgga2V5IGluc2lnaHRzIG9uIHBvc3NpYmxlIGNvcGluZyByZXNwb25zZXMgYW5kIGFkYXB0aXZlIGNhcGFjaXR5IG5lY2Vzc2FyeSBmb3IgdGhlIHNtYWxsLXNjYWxlIGZpc2hlcmllcyBjb21tdW5pdGllcyB0byB0cmFuc2l0aW9uIHRvd2FyZCB2aWFiaWxpdHkuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjMzODkvZm1hcnMuMjAyMy4xMDg3Mjk2IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5mcm9udGllcnNpbi5vcmcvYXJ0aWNsZXMvMTAuMzM4OS9mbWFycy4yMDIzLjEwODcyOTYvcGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjMtMDYtMjkiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTEwIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJGcm9udGllcnMgaW4gTWFyaW5lIFNjaWVuY2UiLCJ2b2x1bWUiOjEwLCJpbXBhY3RGYWN0b3IiOiI0LjI4In0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkZyb250aWVycyBNZWRpYSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NTEsImNpdGVkQnlDb3VudCI6NX19LCJxdW90ZSI6IldhdGVyIHF1YWxpdHkgb2YgQ2hpbGlrYSBMYWdvb24gaGFzIGRlZ3JhZGVkIGR1ZSB0byBhbnRocm9wb2dlbmljIGludGVydmVudGlvbnMgYW5kIG5hdHVyYWwgZm9yY2VzIG9mIGNoYW5nZS4gVmFyaW91cyBhY3Rpdml0aWVzIGluIHRoZSBDaGlsaWthIExhZ29vbiwgc3VjaCBhcyBpbmR1c3RyaWFsIHdhc3Rld2F0ZXIgZGlzcG9zYWwsIHNld2FnZSBkdW1waW5nLCBhcXVhY3VsdHVyZSwgaHlkcm9sb2dpY2FsIGludGVydmVudGlvbnMsIGFuZCBjeWNsb25lcywgYXJlIHByb2R1Y2luZyBzYWxpbml0eSBjaGFuZ2VzLCBzZWRpbWVudCBkZXBvc2l0aW9uLCBudXRyaWVudCBlbnJpY2htZW50LCBldXRyb3BoaWNhdGlvbiwgYW5kIGRlYWQgem9uZXMsIGFzIHN0YXRlZC4gRmlzaCBzdXJ2aXZhbCBpcyBhdCByaXNrIGFzIGEgcmVzdWx0IG9mIHRoZSBzaWduaWZpY2FudCByZWR1Y3Rpb24gaW4gZW52aXJvbm1lbnRhbCBjaXJjdW1zdGFuY2VzLCB3aGljaCBkaW1pbmlzaGVzIGZpc2hlcm1lbuKAmXMgaW5jb21lIGFuZCBwdXNoZXMgdGhlbSBvdXQgb2Ygc29jaWV0eS4gVGhlc2UgY2F1c2VzIGFmZmVjdGluZyB0aGUgd2F0ZXIgcXVhbGl0eSBvZiB0aGUgTGFnb29uIGRlcGljdCBtYW55IGZhY2VzIG9mIHZ1bG5lcmFiaWxpdHkgaW4gU1NGIGNvbW11bml0aWVzLiBBcyB0aGUgQ2hpbGlrYSBMYWdvb24gZW52aXJvbm1lbnQgaXMgZXhwZWN0ZWQgdG8gdW5kZXJnbyBpbmNyZWFzaW5nbHkgZHJhc3RpYyBjaGFuZ2VzIChzdWNoIGFzIGNhdGNoIGNvbXBvc2l0aW9uLCBjYXRjaCBjYXBhY2l0eSwgYmlvZGl2ZXJzaXR5IHZhcmlhdGlvbnMsIHdhdGVyIHF1YWxpdHksIGFuZCBmaXNoZXJ5IHJldmVudWVzKSwgaW50ZXJuYXRpb25hbCBwb2xpY3kgaGFzIGJlY29tZSBtb3JlIGludGVyZXN0ZWQgaW4gYWRhcHRhdGlvbiBtZWFzdXJlcy4gR2l2ZW4gaG93IGRpdmVyc2UgU1NGIGVjb3N5c3RlbXMgYXJlIGV4cGVjdGVkIHRvIGJlIGJldHdlZW4gY29udGluZW50cywgdGhlcmUgaXMgbm8g4oCcb25lIHNpemUgZml0cyBhbGzigJ0gYXBwcm9hY2guIFRoZSBwYXBlciBvdXRsaW5lZCBzZXZlcmFsIGNvcGluZyBhbmQgYWRhcHRhdGlvbiBtZWNoYW5pc21zIHRvIGltcHJvdmUgd2F0ZXIgcXVhbGl0eSBhbmQgY29udGludWUgZmlzaGluZyBpbiBhIHN1c3RhaW5hYmxlIHdheS4gQWRhcHRhdGlvbiBzdHJhdGVnaWVzIGNhbiBvY2Nhc2lvbmFsbHkgaGF2ZSB1bmZvcmVzZWVuIGNvbnNlcXVlbmNlcyBmb3IgZmlzaGVyaWVzIGFuZCBmaXNoaW5nIGNvbW11bml0aWVzLiBBbnkgdW5leHBlY3RlZCBjb25zZXF1ZW5jZXMgb2YgYWRhcHRhdGlvbiBjYW4gYmUgbWl0aWdhdGVkIHRocm91Z2ggbG9uZy10ZXJtIHBsYW5uaW5nIGFuZCBpZGVudGlmaWNhdGlvbiBvZiBmdXR1cmUgb3B0aW9ucyB0aHJvdWdoIHNjZW5hcmlvIGFuYWx5c2lzLiBUaGUgZmluZGluZ3Mgb2YgdGhpcyBwYXBlciBjYW4gYmUgdXRpbGl6ZWQgdG8gZGV2ZWxvcCBwb2xpY2llcyB0byBwcm90ZWN0IHdhdGVyIHF1YWxpdHkgYW5kIFNTRnMgaW4gYSB2YXJpZXR5IG9mIGNvYXN0YWwgbGFnb29uIHNldHRpbmdzLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyMi4yODUwMzYwODcwMzYxMzMsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40OTMwNTM0MzYyNzkzLCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyMy43NzM1MTk1MTU5OTEyMSwibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ4MzM2NDEwNTIyNDYxLCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyNS4yNTU1ODQ3MTY3OTY4NzUsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40ODk1NTkxNzM1ODM5ODQsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjI2Ljc0NDA2ODE0NTc1MTk1MywibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjUwNjgwNTQxOTkyMTg3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjguMjI2MTMzMzQ2NTU3NjE3LCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDkwNDYzMjU2ODM1OTQsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjI5LjcxNDYxNjc3NTUxMjY5NSwibGVmdCI6OC4zMzE1OTI1NTk4MTQ0NTMsIndpZHRoIjo0MC40OTQwOTg2NjMzMzAwOCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MzEuMTk2NjgzODgzNjY2OTkyLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDk0MzkyMzk1MDE5NTMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjMyLjY4NTE2NTQwNTI3MzQ0LCJsZWZ0Ijo4LjMzMTU5NjM3NDUxMTcxOSwid2lkdGgiOjQwLjQ4NzQ0NTgzMTI5ODgzLCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozNC4xNjcyMzI1MTM0Mjc3MzQsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDk3NzQ1NTEzOTE2MDE2LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozNS42NTU3MTU5NDIzODI4MSwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40OTg2MzgxNTMwNzYxNywiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MzcuMTM3NzgzMDUwNTM3MTEsImxlZnQiOjEwLjY4NTE4NjM4NjEwODM5OCwid2lkdGgiOjM4LjE0MTEwNTY1MTg1NTQ3LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozOC42MjYyNjY0Nzk0OTIxOSwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40ODcxNTIwOTk2MDkzNzUsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQwLjEwODMzMzU4NzY0NjQ4NCwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40ODk4NTY3MTk5NzA3LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo0MS41OTY4MTMyMDE5MDQzLCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4ODY2NjUzNDQyMzgzLCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo0My4wNzg4ODAzMTAwNTg1OTQsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDgxODYxMTE0NTAxOTUsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQ0LjU2NzM2MzczOTAxMzY3LCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4MzA1ODkyOTQ0MzM2LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo0Ni4wNDk0MzA4NDcxNjc5NywibGVmdCI6OC4zMzMxMDg5MDE5Nzc1MzksIndpZHRoIjo0MC41MDA3NDc2ODA2NjQwNiwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDcuNTM3OTE0Mjc2MTIzMDUsImxlZnQiOjguMzMzMTA4OTAxOTc3NTM5LCJ3aWR0aCI6NDAuNDk1NjA1NDY4NzUsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQ5LjAxOTk4MTM4NDI3NzM0NCwibGVmdCI6OC4zMzMxMDYwNDA5NTQ1OSwid2lkdGgiOjQwLjUwNjgxNjg2NDAxMzY3LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo1MC41MDg0NjQ4MTMyMzI0MiwibGVmdCI6OC4zMzMxMDYwNDA5NTQ1OSwid2lkdGgiOjQwLjQ4ODUwNjMxNzEzODY3LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo1MS45OTA1MjgxMDY2ODk0NSwibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ5NzcyNjQ0MDQyOTY5LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo1My40NzkwMTE1MzU2NDQ1MywibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjMzLjE1OTQ0NjcxNjMwODU5NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NTQuOTYxMDc4NjQzNzk4ODMsImxlZnQiOjEwLjY4MzY3MDA0Mzk0NTMxMiwid2lkdGgiOjM4LjEzNjU5NjY3OTY4NzUsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjU2LjQ0OTU2MjA3Mjc1MzkwNiwibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ4NzE0NDQ3MDIxNDg0NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIxMyJ9fV0%3D. Similarly, Bengaluru residents could form networks around Bellandur, reporting foam plumes from surfactant-rich sewage a key bloom trigger (Gobler et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkNocmlzdG9waGVyIEouIiwiZmFtaWx5IjoiR29ibGVyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlJ1dGggVy4iLCJmYW1pbHkiOiJEcmlua3dhdGVyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkEuIFcuIiwiZmFtaWx5IjoiQW50aG9ueSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJKZW5uaWZlciBBLiIsImZhbWlseSI6IkdvbGVza2kifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW5uIE1hcmllIEUuIiwiZmFtaWx5IjoiRmFtdWxhcm8tUGVjb3JhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1pY2hhZWwgQi4iLCJmYW1pbHkiOiJXYWxsYWNlIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5vcmEgUi4gVy4iLCJmYW1pbHkiOiJTdHJhcXVhZGluZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJSb25vam95IiwiZmFtaWx5IjoiSGVtIn19XSwiaWRzIjp7Imlzc24iOlsiMTY2NC0zMDJYIl0sImlzYm4iOltdLCJkb2kiOiIxMC4zMzg5L2ZtaWNiLjIwMjQuMTQ2NDY4NiIsInBtaWQiOiIzOTU2ODk5NyIsIm9wZW5hbGV4IjoiVzQ0MDQxMjA5NTIiLCJleGEiOiJodHRwczovL3d3dy5mcm9udGllcnNpbi5vcmcvam91cm5hbHMvbWljcm9iaW9sb2d5L2FydGljbGVzLzEwLjMzODkvZm1pY2IuMjAyNC4xNDY0Njg2L2Z1bGwifSwidGl0bGUiOiJTZXdhZ2UtYW5kIGZlcnRpbGl6ZXItZGVyaXZlZCBudXRyaWVudHMgYWx0ZXIgdGhlIGludGVuc2l0eSwgZGl2ZXJzaXR5LCBhbmQgdG94aWNpdHkgb2YgaGFybWZ1bCBjeWFub2JhY3RlcmlhbCBibG9vbXMgaW4gZXV0cm9waGljIGxha2VzIiwiYWJzdHJhY3QiOiJDeWFub2JhY3RlcmlhbCBoYXJtZnVsIGFsZ2FsIGJsb29tcyAoQ0hBQnMpIGFyZSBwcm9tb3RlZCBieSBleGNlc3NpdmUgbnV0cmllbnQgbG9hZGluZyBhbmQsIHdoaWxlIGZlcnRpbGl6ZXJzIGFuZCBzZXdhZ2UgYXJlIHRoZSBtb3N0IHByZXZhbGVudCBleHRlcm5hbCBudXRyaWVudCBzb3VyY2VzIGluIG1vc3Qgd2F0ZXJzaGVkcywgdGhlIGRpZmZlcmVudGlhbCBlZmZlY3RzIG9mIHRoZXNlIG51dHJpZW50IHNvdXJjZXMgb24gQ0hBQnMgYXJlIHVua25vd24uIEhlcmUsIHdlIHRyYWNrZWQgQ0hBQnMgYW5kIHBlcmZvcm1lZCBleHBlcmltZW50cyBpbiBmaXZlIGRpc3RpbmN0IGxha2VzIGFjcm9zcyB0aGUgTm9ydGhlcm4gVVMgaW5jbHVkaW5nIExha2UgRXJpZS4gRmVydGlsaXplcnMgd2l0aCBhbW1vbml1bSBhbmQgb3J0aG9waG9zcGhhdGUsIG1lbWJyYW5lICgwLjIgzrxtKS1maWx0ZXJlZCBzZXdhZ2UgKGRvbWluYXRlZCBieSByZWR1Y2VkIGZvcm1zIG9mIG5pdHJvZ2VuKSBzYW5kLWFuZCBtZW1icmFuZS1maWx0ZXJlZCBzZXdhZ2UgKGRvbWluYXRlZCBieSBuaXRyYXRlKSwgYW5kIGFuIGlub3JnYW5pYyBudXRyaWVudCBzb2x1dGlvbiBvZiBhbW1vbml1bSBhbmQgb3J0aG9waG9zcGhhdGUgd2VyZSB1c2VkIGFzIGV4cGVyaW1lbnRhbCBudXRyaWVudCBzb3VyY2VzIGZvciBDSEFCcyBhdCBOLWVxdWl2YWxlbnQsIGVudmlyb25tZW50YWxseSByZWFsaXN0aWMgY29uY2VudHJhdGlvbnMuIFBoeXRvcGxhbmt0b24gY29tbXVuaXRpZXMgd2VyZSBldmFsdWF0ZWQgZmx1b3JvbWV0cmljYWxseSwgbWljcm9zY29waWNhbGx5LCBhbmQgdmlhIGhpZ2ggdGhyb3VnaHB1dCBzZXF1ZW5jaW5nIG9mIHRoZSAxNlMgclJOQSBnZW5lLCBhbmQgbGV2ZWxzIG9mIG1pY3JvY3lzdGluIGFuZCB0aGUgzrQgMTUgTiBjb250ZW50IG9mIHBhcnRpY3VsYXRlIG9yZ2FuaWMgbml0cm9nZW4gKM60UE8gMTUgTikgd2VyZSBxdWFudGlmaWVkLiBGZXJ0aWxpemVyIGFuZCBib3RoIHNvdXJjZXMgb2Ygd2FzdGV3YXRlciBpbmNyZWFzZWQgdGhlIGFidW5kYW5jZSBvZiBjeWFub2JhY3RlcmlhIGluIGFsbCBleHBlcmltZW50cyBhY3Jvc3MgYWxsIGZpdmUgbGFrZXMgKCBwICZhbXA7bHQ7IDAuMDUgZm9yIGFsbCkgd2hlcmVhcyBlZmZlY3RzIG9uIGV1a2FyeW90aWMgcGh5dG9wbGFua3RvbiB3ZXJlIGxpbWl0ZWQuIFNhbmQtZmlsdGVyZWQgc2V3YWdlIGNvbnRhaW5lZCBsZXNzIFAsIG9yZ2FuaWMgbWF0dGVyLCBhbmQgYW1tb25pdW0gYnV0IG1vcmUgbml0cmF0ZSBhbmQgaGFkIGEgMjUlIGxlc3MgcG90ZW50IHN0aW11bGF0b3J5IGVmZmVjdCBvbiBjeWFub2JhY3RlcmlhIHRoYW4gbWVtYnJhbmUtZmlsdGVyZWQgc2V3YWdlLCBzdWdnZXN0aW5nIG5pdHJpZmljYXRpb24gbWF5IHBsYXkgYSByb2xlIGluIHJlZHVjaW5nIENIQUJzLiBGZXJ0aWxpemVyIGluY3JlYXNlZCBtaWNyb2N5c3RpbiBsZXZlbHMgYW5kIGRlY3JlYXNlZCB0aGUgzrRQTyAxNSBOIHdoZXJlYXMgd2FzdGV3YXRlciBpbmNyZWFzZWQgzrRQTyAxNSBOICggcCAmYW1wO2x0OyAwLjA1IGZvciBhbGwpLiBNaWNyb2N5c3RpcyB3YXMgdGhlIGdlbnVzIG1vc3QgY29uc2lzdGVudGx5IHByb21vdGVkIGJ5IG51dHJpZW50IHNvdXJjZXMgKCBwICZhbXA7bHQ7IDAuMDUgaW4gYWxsIGV4cGVyaW1lbnRzKSwgZm9sbG93ZWQgYnkgQ3lhbm9iaXVtICggcCAmYW1wO2x0OyAwLjA1IGluIDUwJSBvZiBleHBlcmltZW50cyksIHdpdGggaW5jcmVhc2VzIGluIE1pY3JvY3lzdGlzIGJpb21hc3MgY29uc2lzdGVudGx5IGVsaWNpdGVkIGJ5IG1lbWJyYW5lLWZpbHRlcmVkIHdhc3Rld2F0ZXIuIENvbGxlY3RpdmVseSwgcmVzdWx0cyBkZW1vbnN0cmF0ZSB0aGF0IGRpZmZlcmluZyB0eXBlcyBvZiBzZXdhZ2UgZGlzY2hhcmdlIGFuZCBmZXJ0aWxpemVycyBjYW4gcHJvbW90ZSBDSEFCIGludGVuc2l0eSBhbmQgdG94aWNpdHksIHdoaWxlIGNvbmN1cnJlbnRseSBhbHRlcmluZyBDSEFCIGRpdmVyc2l0eSBhbmQgzrRQTyAxNSBOLiBXaGlsZSBtZW1icmFuZS1maWx0ZXJlZCBzZXdhZ2UgY29uc2lzdGVudGx5IGZhdm9yZWQgTWljcm9jeXN0aXMgLCB0aGUgZGlzY2hhcmdlIG9mIHNld2FnZSB0aHJvdWdoIHNhbmRzIG11dGVkIGJsb29tIGludGVuc2l0eSBzdWdnZXN0aW5nIHNhbmQtYmVkcyBtYXkgcmVwcmVzZW50IGEgdG9vbCB0byByZW1vdmUga2V5IG51dHJpZW50cyBhbmQgcGFydGlhbGx5IG1pdGlnYXRlIENIQUJzLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzg5L2ZtaWNiLjIwMjQuMTQ2NDY4NiIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cuZnJvbnRpZXJzaW4ub3JnL2pvdXJuYWxzL21pY3JvYmlvbG9neS9hcnRpY2xlcy8xMC4zMzg5L2ZtaWNiLjIwMjQuMTQ2NDY4Ni9wZGYiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMS0wNiIsImFjY2Vzc2VkRGF0ZSI6IjIwMjYtMDIiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZyb250aWVycyBpbiBNaWNyb2Jpb2xvZ3kiLCJ2b2x1bWUiOjE1LCJpbXBhY3RGYWN0b3IiOiIzLjk5In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTQ2NDY4NiwibGFzdFBhZ2UiOjE0NjQ2ODZ9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRnJvbnRpZXJzIE1lZGlhIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo3MywiY2l0ZWRCeUNvdW50Ijo5fX0sInF1b3RlIjoiZGVyaXZlZCBudXRyaWVudHMgYWx0ZXIgdGhlIGludGVuc2l0eSwgZGl2ZXJzaXR5LCBhbmQgdG94aWNpdHkgb2YgaGFybWZ1bCBjeWFub2JhY3RlcmlhbCBibG9vbXMgLi4uIEN5YW5vYmFjdGVyaWFsIGhhcm1mdWwgYWxnYWwgYmxvb21zIChDSEFCcykgYXJlIHByb21vdGVkIGJ5IGV4Y2Vzc2l2ZSBudXRyaWVudCBsb2FkaW5nIGFuZCwgd2hpbGUgZmVydGlsaXplcnMgYW5kIHNld2FnZSBhcmUgdGhlIG1vc3QgcHJldmFsZW50IGV4dGVybmFsIG51dHJpZW50IHNvdXJjZXMgaW4gbW9zdCB3YXRlcnNoZWRzLCB0aGUgZGlmZmVyZW50aWFsIGVmZmVjdHMgb2YgdGhlc2UgbnV0cmllbnQgLi4uIG9yZ2FuaWMgbml0cm9nZW4gKM60UE8xNU4pIHdlcmUgcXVhbnRpZmllZC4gRmVydGlsaXplciBhbmQgYm90aCBzb3VyY2VzIG9mIHdhc3Rld2F0ZXIgaW5jcmVhc2VkIHRoZSBhYnVuZGFuY2Ugb2YgLi4uIHBoeXRvcGxhbmt0b24gd2VyZSBsaW1pdGVkLiBTYW5kLWZpbHRlcmVkIHNld2FnZSBjb250YWluZWQgbGVzcyBQLCBvcmdhbmljIG1hdHRlciwgYW5kIGFtbW9uaXVtIGJ1dCBtb3JlIG5pdHJhdGUgYW5kIC4uLiBkaWZmZXJpbmcgdHlwZXMgb2Ygc2V3YWdlIGRpc2NoYXJnZSBhbmQgZmVydGlsaXplcnMgY2FuIHByb21vdGUgQ0hBQiBpbnRlbnNpdHkgYW5kIHRveGljaXR5LCB3aGlsZSBjb25jdXJyZW50bHkgLi4uIHNld2FnZSB0aHJvdWdoIHNhbmRzIG11dGVkIGJsb29tIGludGVuc2l0eSBzdWdnZXN0aW5nIHNhbmQtYmVkcyBtYXkgcmVwcmVzZW50IGEgdG9vbCB0byByZW1vdmUga2V5IG51dHJpZW50cyBhbmQgLi4uICggV2F0c29uIGV0IGFsLiwgMjAxNiA7IEJ1bGxlcmphaG4gZXQgYWwuLCAyMDE2IDsgSHVpc21hbiBldCBhbC4sIDIwMTggKSBhbmQgZXhjZXNzaXZlIG51dHJpZW50IGxvYWRpbmcgaXMgdGhlIHByb2Nlc3MgbW9zdCBjb21tb25seSBjaXRlZCBhcyB0aGUgZW52aXJvbm1lbnRhbCBmYWN0b3IgcHJvbW90aW5nIENIQUJzICgifV0%3D. Global scans affirm that non-governmental organizations and volunteers worldwide employ such strategies, yielding cost-effective early warnings that outpace static best management practices, especially under climate stressors like prolonged stratification (Meerhoff et al., 2022; Paerl et al., 2019)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik0uIiwiZmFtaWx5IjoiTWVlcmhvZmYifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSi4iLCJmYW1pbHkiOiJBdWRldCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJULkEuIiwiZmFtaWx5IjoiRGF2aWRzb24ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTC4gRGUiLCJmYW1pbHkiOiJNZWVzdGVyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlMuIiwiZmFtaWx5IjoiSGlsdCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTLiIsImZhbWlseSI6Iktvc3RlbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJaYWlodWEiLCJmYW1pbHkiOiJMaXUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTi4iLCJmYW1pbHkiOiJNYXp6ZW8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSGFucyBXLiIsImZhbWlseSI6IlBhZXJsIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik0uIiwiZmFtaWx5IjoiU2NoZWZmZXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiRS4iLCJmYW1pbHkiOiJKZXBwZXNlbiJ9fV0sImlkcyI6eyJpc3NuIjpbXSwiaXNibiI6W10sImRvaSI6IjEwLjE3NjE1Lzd2ZTUtdzAxNiIsIm9wZW5hbGV4IjoiVzQzMTg0MTU1NzcifSwidGl0bGUiOiJGZWVkYmFjayBiZXR3ZWVuIGNsaW1hdGUgY2hhbmdlIGFuZCBldXRyb3BoaWNhdGlvbjogcmV2aXNpdGluZyB0aGUgYWxsaWVkIGF0dGFjayBjb25jZXB0IGFuZCBob3cgdG8gc3RyaWtlIGJhY2siLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTc2MTUvN3ZlNS13MDE2IiwicmVzb3VyY2VVcmwiOiJodHRwczovL2Nkci5saWIudW5jLmVkdS9kb3dubG9hZHMvaDk4OXJjOTM4IiwiaXNzdWVkRGF0ZSI6IjIwMjItMDEtMDEiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTEwIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJDYXJvbGluYSBEaWdpdGFsIFJlcG9zaXRvcnkgKFVuaXZlcnNpdHkgb2YgTm9ydGggQ2Fyb2xpbmEgYXQgQ2hhcGVsIEhpbGwpIiwiaW1wYWN0RmFjdG9yIjoiMC4xMCJ9LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJVbml2ZXJzaXR5IG9mIE5vcnRoIENhcm9saW5hIGF0IENoYXBlbCBIaWxsIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjowLCJjaXRlZEJ5Q291bnQiOjN9fSwicXVvdGUiOiJXaXRoIGNsaW1hdGUgd2FybWluZywgdGhlIGRpcmVjdCBhbmQgaW5kaS1yZWN0IGVmZmVjdHMgb2YgaW5jcmVhc2VkIGxha2UgdGVtcGVyYXR1cmVzIHdpbGwgbGlrZWx5IGRyaXZlIGNoYW5nZXMgaW4gY3lhbm9iYWN0ZXJpYWwgdGF4YSBhbmQgdGhlaXIgcHJvZHVjLXRpb24gb2YgY3lhbm9iYWN0ZXJpYWwgdG94aW5zLCBsaWtlbHkgaW5jbHVkaW5nIHByb21vdC1pbmcgdGhlIHNlbGVjdGlvbiBvZiBhIGZldyBNaWNyb2N5c3RpcyBzcGVjaWVzIG9yIHN0cmFpbnMgcHJvZHVjaW5nIGhpZ2hseSB0b3hpYyBtaWNyb2N5c3Rpbi1MUiAoTWFudHpvdWtpIGV0IGFsLiAyMDE4KS4gRGVzcGl0ZSB2YXJpYXRpb25zIGFtb25nIGxha2VzIGluIGEgd2F0ZXItc2NhcGUsIGEgcmVnaW9uYWwgaW5jcmVhc2UgaW4gbWljcm9jeXN0aW4gbWF4aW1hIGFuZCBkdXJhdGlvbiBvZiBibG9vbXMgaGFzIGFsc28gYmVlbiBjbGVhcmx5IGV4cGxhaW5lZCBieSBpbmNyZWFzaW5nIHRlbXBlcmF0dXJlIGluIGNlbnRyYWwgTm9ydGggQW1lcmljYSAoSGF5ZXMgZXQgYWwuIDIwMjApLCB3aXRoIGluY3JlYXNpbmcgcmlza3MgdG8gaHVtYW4gaGVhbHRoLiBHcm93aW5nIGV2aWRlbmNlIGluZGljYXRlcyB0aGF0IGN5YW5vYmFjdGVyaWFsIGRvbWluYW5jZSB3aWxsIGV4ZXJ0IGEgcG9zaXRpdmUgZmVlZGJhY2sgd2l0aCBjbGltYXRlIHdhcm1pbmcgKEhhdmVucyBldCBhbC4gMjAxNiwgUGFlcmwgZXQgYWwuIDIwMjApLiBNYW55IGN5YW5vYmFjdGVyaWFsIHNwZWNpZXMgcHJvbGlmZXJhdGUgb24gdGhlIGxha2Ugc3VyZmFjZSwgYWJzb3JiaW5nIGxpZ2h0LCBlbmhhbmNpbmcgbG9jYWxpemVkIHRlbXBlcmF0dXJlIGluY3JlYXNlcywgYW5kIGluY3JlYXNpbmcgdGhlcm1hbCBzdHJhdGlmaWNhdGlvbiBwb3RlbnRpYWwgKEthaHJ1IGV0IGFsLiAxOTkzKS4gQ3lhbm9iYWN0ZXJpYSBwcm9tb3RlIHRoZSByZWxlYXNlIG9mIEdIR3MsIHBhcnRpY3VsYXJseSBDSDQgKFlhbiBldCBhbC4gMjAxNyksIGR1cmluZyB0aGUgZGVjb21wb3NpdGlvbiBmb2xsb3dpbmcgdGhlIGNvbGxhcHNlIG9mIGEgYmxvb20uIEN5YW5vYmFjdGVyaWEgYWxzbyBwcm9tb3RlIHJlbWluZXJhbGl6YXRpb24gb2YgbnV0cmktZW50cyBkdXJpbmcgYmxvb21zLCBsZWFkaW5nIHRvIG11bHRpcGxlIGJsb29tcyBpbiBhIHNpbmdsZSBzZWFzb24uIFVuZGVyIGV1dHJvcGhpYyBhbmQgd2FybWluZyBjb25kaS10aW9ucywgY3lhbm9iYWN0ZXJpYSBkb21pbmFuY2UgbWF5IHRodXMgYmUgYSBzZWxmLXBlcnBldHVhdGluZyBwcm9waGVjeSAoRmlnLiAyIGFuZCAzKS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo2Ny4wNjM0MDAyNjg1NTQ2OSwibGVmdCI6NTEuNDc4ODAxNzI3Mjk0OTIsIndpZHRoIjozOC42NzM0MjM3NjcwODk4NDQsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6NjguNjk4NTQ3MzYzMjgxMjUsImxlZnQiOjUxLjQ3ODgwMTcyNzI5NDkyLCJ3aWR0aCI6MzguNjc5NDk2NzY1MTM2NzIsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6NzAuMzMzNjg2ODI4NjEzMjgsImxlZnQiOjUxLjQ3ODgwMTcyNzI5NDkyLCJ3aWR0aCI6MzguNjc3NTgxNzg3MTA5Mzc1LCJoZWlnaHQiOjEuNDcxMTAxNTIyNDQ1Njc4N319LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjcxLjk2ODgyNjI5Mzk0NTMxLCJsZWZ0Ijo1MS40Nzg4MDE3MjcyOTQ5Miwid2lkdGgiOjM4LjY3OTEwMzg1MTMxODM2LCJoZWlnaHQiOjEuNDcxMTAxNTIyNDQ1Njc4N319LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjczLjYxMDU3MjgxNDk0MTQsImxlZnQiOjUxLjQ3ODgwMTcyNzI5NDkyLCJ3aWR0aCI6MzguNjc0NDI3MDMyNDcwNywiaGVpZ2h0IjoxLjQ3Mzc0Njg5NTc5MDF9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo3NS4yNDcwMzk3OTQ5MjE4OCwibGVmdCI6NTEuNDc4ODA5MzU2Njg5NDUsIndpZHRoIjozOC42NzQyNjMwMDA0ODgyOCwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo3Ni44ODIxODY4ODk2NDg0NCwibGVmdCI6NTEuNDc4ODA5MzU2Njg5NDUsIndpZHRoIjozOC42ODExNzE0MTcyMzYzMywiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo3OC41MjUyNTMyOTU4OTg0NCwibGVmdCI6NTEuNDc4ODA5MzU2Njg5NDUsIndpZHRoIjozOC42NzU2NjI5OTQzODQ3NjYsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODAuMTYwNDAwMzkwNjI1LCJsZWZ0Ijo1MS40Nzg4MDkzNTY2ODk0NSwid2lkdGgiOjM4LjY4MjAyOTcyNDEyMTA5NCwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo4MS43OTU1Mzk4NTU5NTcwMywibGVmdCI6NTEuNDc4ODA5MzU2Njg5NDUsIndpZHRoIjozOC42NzQ5ODM5NzgyNzE0ODQsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODMuNDM4NjEzODkxNjAxNTYsImxlZnQiOjUxLjQ3ODgwOTM1NjY4OTQ1LCJ3aWR0aCI6MzguNjc2MTg1NjA3OTEwMTU2LCJoZWlnaHQiOjEuNDg2OTczNTI0MDkzNjI4fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODUuMDg5NjMwMTI2OTUzMTIsImxlZnQiOjUxLjQ3ODgwOTM1NjY4OTQ1LCJ3aWR0aCI6NC42ODI4MDUwNjEzNDAzMzIsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODYuNzI0Nzc3MjIxNjc5NjksImxlZnQiOjUzLjQ0MTE3NzM2ODE2NDA2LCJ3aWR0aCI6Ni4xNDAzNTU1ODcwMDU2MTUsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODYuNzI0ODkxNjYyNTk3NjYsImxlZnQiOjYwLjk5NDg3Njg2MTU3MjI2Niwid2lkdGgiOjYuMDM2OTkyNTQ5ODk2MjQsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODYuNzI0ODkxNjYyNTk3NjYsImxlZnQiOjY4LjQ0NjUyNTU3MzczMDQ3LCJ3aWR0aCI6Ni4xNTA3MDU4MTQzNjE1NzIsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODYuNzI0ODkxNjYyNTk3NjYsImxlZnQiOjc2LjAwOTgyNjY2MDE1NjI1LCJ3aWR0aCI6Mi43MjczMzMzMDcyNjYyMzU0LCJoZWlnaHQiOjEuNDcxMTAxNTIyNDQ1Njc4N319LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjg2LjcyNDg5MTY2MjU5NzY2LCJsZWZ0Ijo4MC4xNDk1ODE5MDkxNzk2OSwid2lkdGgiOjEwLjAwODIzNDk3NzcyMjE2OCwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo4OC4zNjAwMzg3NTczMjQyMiwibGVmdCI6NTEuNDc3NjU3MzE4MTE1MjM0LCJ3aWR0aCI6MzguNjcyMDU0MjkwNzcxNDg0LCJoZWlnaHQiOjEuNDcxMTAxNTIyNDQ1Njc4N319LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjkwLjAwMzExMjc5Mjk2ODc1LCJsZWZ0Ijo1MS40Nzc2NTczMTgxMTUyMzQsIndpZHRoIjozOC42NzY0OTg0MTMwODU5NCwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo5MS42MzgyNTIyNTgzMDA3OCwibGVmdCI6NTEuNDc3NjUzNTAzNDE3OTcsIndpZHRoIjozOC42NzU4MzQ2NTU3NjE3MiwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjksImJib3giOnsidG9wIjo2LjQ2ODkzNTk2NjQ5MTY5OSwibGVmdCI6OS44NDAxMzM2NjY5OTIxODgsIndpZHRoIjozOC42ODYzMjUwNzMyNDIxOSwiaGVpZ2h0IjoxNi4yMTIxMjU3NzgxOTgyNDJ9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI5In19LHsib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhhbnMgVy4iLCJmYW1pbHkiOiJQYWVybCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLYXJsIEUuIiwiZmFtaWx5IjoiSGF2ZW5zIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hdGhhbiBTLiIsImZhbWlseSI6IkhhbGwifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiVGltb3RoeSBHLiIsImZhbWlseSI6Ik90dGVuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1lbmd5dWFuIiwiZmFtaWx5IjoiWmh1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhhaSIsImZhbWlseSI6Ilh1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikd1YW5nd2VpIiwiZmFtaWx5IjoiWmh1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkJvcWlhbmciLCJmYW1pbHkiOiJRaW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIxMzIzLTE2NTAiLCIxNDQ4LTYwNTkiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwNzEvbWYxODM5MiIsIm1hZyI6IjI5NDIzMjE1NDIiLCJvcGVuYWxleCI6IlcyOTQyMzIxNTQyIn0sInRpdGxlIjoiTWl0aWdhdGluZyBhIGdsb2JhbCBleHBhbnNpb24gb2YgdG94aWMgY3lhbm9iYWN0ZXJpYWwgYmxvb21zOiBjb25mb3VuZGluZyBlZmZlY3RzIGFuZCBjaGFsbGVuZ2VzIHBvc2VkIGJ5IGNsaW1hdGUgY2hhbmdlIiwiYWJzdHJhY3QiOiJNYW5hZ2luZyBhbmQgbWl0aWdhdGluZyB0aGUgZ2xvYmFsIGV4cGFuc2lvbiBvZiB0b3hpYyBjeWFub2JhY3RlcmlhbCBoYXJtZnVsIGFsZ2FsIGJsb29tcyAoQ3lhbm9IQUJzKSBpcyBhIG1ham9yIGNoYWxsZW5nZSBmYWNpbmcgcmVzZWFyY2hlcnMgYW5kIHdhdGVyIHJlc291cmNlIG1hbmFnZXJzLiBWYXJpb3VzIGFwcHJvYWNoZXMsIGluY2x1ZGluZyBudXRyaWVudCBsb2FkIHJlZHVjdGlvbiwgYXJ0aWZpY2lhbCBtaXhpbmcgYW5kIGZsdXNoaW5nLCBvbW5pdm9yb3VzIGZpc2ggcmVtb3ZhbCwgYWxnYWVjaWRlIGFwcGxpY2F0aW9ucyBhbmQgc2VkaW1lbnQgZHJlZGdpbmcsIGhhdmUgYmVlbiB1c2VkIHRvIHJlZHVjZSBibG9vbSBvY2N1cnJlbmNlcy4gSG93ZXZlciwgbWFuYWdlcnMgbm93IGZhY2UgdGhlIGFkZGl0aW9uYWwgY2hhbGxlbmdlIG9mIGhhdmluZyB0byBhZGRyZXNzIHRoZSBlZmZlY3RzIG9mIGNsaW1hdGUgY2hhbmdlIG9uIHdhdGVyc2hlZCBoeWRyb2xvZ2ljYWwgYW5kIG51dHJpZW50IGxvYWQgZHluYW1pY3MsIHdhdGVyIHRlbXBlcmF0dXJlLCBtaXhpbmcgcmVnaW1lIGFuZCBpbnRlcm5hbCBudXRyaWVudCBjeWNsaW5nLiBSaXNpbmcgdGVtcGVyYXR1cmVzIGFuZCBpbmNyZWFzaW5nIGZyZXF1ZW5jaWVzIGFuZCBtYWduaXR1ZGVzIG9mIGV4dHJlbWUgd2VhdGhlciBldmVudHMsIGluY2x1ZGluZyB0cm9waWNhbCBjeWNsb25lcywgZXh0cmF0cm9waWNhbCBzdG9ybXMsIGZsb29kcyBhbmQgZHJvdWdodHMsIGFsbCBwcm9tb3RlIEN5YW5vSEFCcyBhbmQgYWZmZWN0IHRoZSBlZmZpY2FjeSBvZiBlY29zeXN0ZW0gcmVtZWRpYXRpb24gbWVhc3VyZXMuIFRoZXNlIGNsaW1hdGljIGNoYW5nZXMgd2lsbCBsaWtlbHkgcmVxdWlyZSBzZXR0aW5nIHN0cmljdGVyIG51dHJpZW50IChpbmNsdWRpbmcgYm90aCBuaXRyb2dlbiBhbmQgcGhvc3Bob3J1cykgcmVkdWN0aW9uIHRhcmdldHMgZm9yIGJsb29tIGNvbnRyb2wgaW4gYWZmZWN0ZWQgd2F0ZXJzLiBJbiBhZGRpdGlvbiwgdGhlIGVmZmljYWN5IG9mIGN1cnJlbnRseSB1c2VkIG1ldGhvZHMgdG8gcmVkdWNlIEN5YW5vSEFCcyB3aWxsIG5lZWQgdG8gYmUgcmUtZXZhbHVhdGVkIGluIGxpZ2h0IG9mIHRoZSBzeW5lcmdpc3RpYyBlZmZlY3RzIG9mIGNsaW1hdGUgY2hhbmdlIHdpdGggbnV0cmllbnQgZW5yaWNobWVudC4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTA3MS9tZjE4MzkyIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2Nkci5saWIudW5jLmVkdS9kb3dubG9hZHMvMDU3NDIyODM0IiwiaXNzdWVkRGF0ZSI6IjIwMTktMDMtMjUiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTEwIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJNYXJpbmUgYW5kIEZyZXNod2F0ZXIgUmVzZWFyY2giLCJpc3N1ZSI6NSwidm9sdW1lIjo3MSwiaW1wYWN0RmFjdG9yIjoiMi43MCJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjU3OSwibGFzdFBhZ2UiOjU3OX19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJDU0lSTyBQdWJsaXNoaW5nIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjoxMDQsImNpdGVkQnlDb3VudCI6OTF9fSwicXVvdGUiOiJUaGF0IHJlbGF0aW9uc2hpcCBoYXMgc2VydmVkIGFzIHRoZSBmb3VuZGF0aW9uIGZvciBzZXR0aW5nICBudXRyaWVudCBsb2FkaW5nIHRhcmdldHMgaW4gbGFrZXMgYXJvdW5kIHRoZSB3b3JsZCwgYW5kIGNhcmVmdWxseSAgZGVzaWduZWQgZXhwZXJpbWVudHMgbWF5IGVsdWNpZGF0ZSB0byB3aGF0IGV4dGVudCB0ZW1wZXJhdHVyZSAgaW5jcmVhc2UgYWZmZWN0cyB0aGUgdGhyZXNob2xkIGF0IHdoaWNoIHRoZSBmcmVxdWVuY3kgb2Ygb2NjdXIgIHJlbmNlIG9mIEN5YW5vSEFCcyBxdWlja2x5IHJpc2VzIHdpdGggaW5jcmVhc2luZyBOIGFuZCBQLiBDb25jbHVzaW9ucyBSZWNlbnQgc3R1ZGllcyBpbmRpY2F0ZSB0aGF0IHRoZSBzeW5lcmdpc3RpYyBjb21iaW5hdGlvbiBvZiBhbnRocm9wb2dlbmljIG51dHJpZW50IGxvYWRpbmcsIHJpc2luZyB0ZW1wZXJhdHVyZXMsIGVuaGFuY2VkICB2ZXJ0aWNhbCBzdHJhdGlmaWNhdGlvbiwgaW5jcmVhc2VkIHJlc2lkZW5jZSB0aW1lIGFuZCBtb3JlICBleHRyZW1lIGNsaW1hdGljIGNvbmRpdGlvbnMgb3ZlcmFsbCB3aWxsIGZhdm91ciBjeWFub2JhY3RlcmlhbCAgZG9taW5hbmNlIGluIGEgd2lkZSByYW5nZSBvZiBhcXVhdGljIGVjb3N5c3RlbXMuIFRoZSBleHBhbiAgc2lvbiBvZiBDeWFub0hBQnMgaGFzIHNlcmlvdXMgY29uc2VxdWVuY2VzIGZvciBodW1hbiB3YXRlciAgc3VwcGxpZXMsIGZpc2hlcmllcyBhbmQgcmVjcmVhdGlvbmFsIHJlc291cmNlcy4gRWNvc3lzdGVtIHNjYWxlICBwaHlzaWNhbCBjaGVtaWNhbCBiaW90aWMgbWl0aWdhdGlvbiBzdHJhdGVnaWVzIHdpbGwgaGF2ZSB0byAgaW5jb3Jwb3JhdGUgbnV0cmllbnQgKG1vc3Qgb2Z0ZW4gYm90aCBOIGFuZCBQKSBpbnB1dCByZWR1Y3Rpb25zLiAgVGhlIG1hZ25pdHVkZXMgb2YgdGhlc2UgcmVkdWN0aW9ucyB3aWxsIG5lZWQgdG8gYmUgc3lzdGVtICBzcGVjaWZpYyBhbmQgd2lsbCBsaWtlbHkgbmVlZCB0byBiZSBhZGp1c3RlZCB3aXRoIGNoYW5naW5nIGNsaSAgbWF0aWMgY29uZGl0aW9ucy4gU3BlY2lmaWNhbGx5LCB3YXRlciBxdWFsaXR5IG1hbmFnZXJzIHdpbGwgaGF2ZSAgdG8gYWNjb21tb2RhdGUgd2FybWluZyBhbmQgZ3JlYXRlciBoeWRyb2xvZ2ljYWwgdmFyaWFiaWxpdHkgIGFuZCBleHRyZW1lbmVzcyByZXN1bHRpbmcgZnJvbSBjbGltYXRlIGNoYW5nZSBpbiBmb3JtdWxhdGluZyAgdGhlaXIgc3RyYXRlZ2llcy4gQSBrZXkgbG9uZyB0ZXJtIGNvbnRyb2wgd2UgY2FuIGV4ZXJ0IHRvIHJlZHVjZSAgdGhlIHJhdGUgYW5kIGV4dGVudCBvZiBnbG9iYWwgd2FybWluZyBpcyBjdXJiaW5nIGdyZWVuaG91c2UgZ2FzICBlbWlzc2lvbnMuIFdpdGhvdXQgdGhpcyBlc3NlbnRpYWwgc3RlcCwgZnV0dXJlIHdhcm1pbmcgdHJlbmRzLCAgaHlkcm9sb2dpY2FsIGV4dHJlbWVzIChyZWNvcmQgcmFpbmZhbGxzIGFuZCBkcm91Z2h0cykgYW5kIHRoZWlyICBlZmZlY3RzIG9uIGFxdWF0aWMgZWNvc3lzdGVtcyB3aWxsIGNvbnRpbnVlIHRvIHByb21vdGUgb3Bwb3IgIHR1bmlzdGljIEN5YW5vSEFCcy4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6MTQuMTQ5MjMzODE4MDU0MiwibGVmdCI6MTMuMTI4NTk1MzUyMTcyODUyLCJ3aWR0aCI6MzcuMDgyMTcyMzkzNzk4ODMsImhlaWdodCI6MS41MTY2MDIyNzc3NTU3MzczfX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjE1LjQ1MTUwODUyMjAzMzY5MSwibGVmdCI6MTMuMTI4NTQ0ODA3NDM0MDgyLCJ3aWR0aCI6MzcuMDg0ODMxMjM3NzkyOTcsImhlaWdodCI6MS41MTY2MDMyMzE0MzAwNTM3fX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjE2Ljc1Mzc4NDE3OTY4NzUsImxlZnQiOjEzLjEyODU0MjkwMDA4NTQ1LCJ3aWR0aCI6MzcuMDgzMjA5OTkxNDU1MDgsImhlaWdodCI6MS41MTY2MDQxODUxMDQzNzAxfX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjE4LjA1NjA1MzE2MTYyMTA5NCwibGVmdCI6MTMuMTI4NTQ0ODA3NDM0MDgyLCJ3aWR0aCI6MzYuNjE1MDc0MTU3NzE0ODQ0LCJoZWlnaHQiOjEuNTE2NjA0MTg1MTA0MzcwMX19LHsicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjoxOS40NjgzNzgwNjcwMTY2LCJsZWZ0IjoxMy4xMjg1NDQ4MDc0MzQwODIsIndpZHRoIjozNC4yOTA1MzExNTg0NDcyNjYsImhlaWdodCI6MS4yNDUwMTEyMTA0NDE1ODk0fX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjIyLjIyNjE0Mjg4MzMwMDc4LCJsZWZ0IjoxMy4xMjg1NTA1Mjk0Nzk5OCwid2lkdGgiOjcuNjQ5ODczNzMzNTIwNTA4LCJoZWlnaHQiOjEuMTM1NDgzMTQ1NzEzODA2Mn19LHsicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjoyMy43MzQ4NDk5Mjk4MDk1NywibGVmdCI6MTMuMTI4NTM1MjcwNjkwOTE4LCJ3aWR0aCI6MzcuMDg5Mjg2ODA0MTk5MjIsImhlaWdodCI6MjYuMDg4NjQwMjEzMDEyNjk1fX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNTg5In19XQ%3D%3D. Training via workshops ensures data quality, with protocols aligning to standards from the National Lake Conservation Plan, transforming passive residents into active sentinels.

The efficacy of these efforts is evident in verifiable outcomes: reduced bloom severity through timely interventions, such as community-led riparian cleanups averting nutrient pulses. In Rewalsar Lake, Himachal Pradesh, grassroots phosphorus tracking could counter legacy recycling under warming conditions (Kumar & Mahajan, 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlBhd2FuIiwiZmFtaWx5IjoiS3VtYXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW1icmlzaCBLdW1hciIsImZhbWlseSI6Ik1haGFqYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyNTIzLTM5NjMiLCIyNTIzLTM5NzEiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMDcvczQyNDUyLTAyMC0zMDgyLTgiLCJtYWciOiIzMDM1OTYwMjI4Iiwib3BlbmFsZXgiOiJXMzAzNTk2MDIyOCJ9LCJ0aXRsZSI6IlRyb3BoaWMgc3RhdHVzIGFuZCBpdHMgcmVndWxhdGluZyBmYWN0b3IgZGV0ZXJtaW5hdGlvbiBhdCB0aGUgUmV3YWxzYXIgTGFrZSwgbm9ydGh3ZXN0IEhpbWFsYXlhIChIUCksIEluZGlhLCBiYXNlZCBvbiBzZWxlY3RlZCBwYXJhbWV0ZXJzIGFuZCBtdWx0aXZhcmlhdGUgc3RhdGlzdGljYWwgYW5hbHlzaXMiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAwNy9zNDI0NTItMDIwLTMwODItOCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9saW5rLnNwcmluZ2VyLmNvbS9jb250ZW50L3BkZi8xMC4xMDA3L3M0MjQ1Mi0wMjAtMzA4Mi04LnBkZiIsImlzc3VlZERhdGUiOiIyMDIwLTA2LTIyIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiU04gQXBwbGllZCBTY2llbmNlcyIsImlzc3VlIjo3LCJ2b2x1bWUiOjIsImltcGFjdEZhY3RvciI6IjcuMzUifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiU3ByaW5nZXIgTmF0dXJlIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo0MCwiY2l0ZWRCeUNvdW50IjoxOX19LCJxdW90ZSI6IkNoYW5naW5nIHF1YWxpdHkgaGFzIGNyZWF0ZWQgdGhlIHdhdGVyIHNjYXJjaXR5IGFuZCAgYSBodWdlIG1hc3Mgb2YgcG9wdWxhdGlvbiB0aHVzIGJlZW4gZGVwcml2ZWQgZnJvbSB1c2luZyAgdGhlIGZyZXNod2F0ZXIgZm9yIHZhcmlvdXMgcHVycG9zZXMgWzMsIDRdLiBIb3dldmVyLCB0aGUgIG9ibm94aW91cyBtYXRlcmlhbHMgdGhhdCBzdGF5IGluIHdhdGVyIGJvZGllcyBmb3IgYSBsb25nLXRpbWUgcGVyaW9kIGJyaW5nIGNoYW5nZXMgaW4gdGhlIHBoeXNpb2NoZW1pY2FsIGNoYXItYWN0ZXJpc3RpY3MgYXMgd2VsbCBhcyB0aGUgbmF0dXJhbCBzY2VuYXJpbyBvZiB0aGUgbGFrZXMgWzVdLiAgVGhlIEhpbWFsYXlhbiBsYWtlcyBhcmUgdW5kZXIgc3RyZXNzIGR1ZSB0byBtaXNtYW5hZ2VkIGh1bWFuIGFjdGl2aXRpZXMsIHdoaWNoIGluY3JlYXNlIG51dHJpZW50cyBhbmQgcG9sbHV0aW9uIGluIHRlcm1zIG9mIGhlYXZ5IG1ldGFscyB0b3hpY2l0eSBhbmQgYmFjdGVyaWFsIGNvbnRhbWluYXRpb24gIFs24oCTOF0uIFRoZSBtYW5hZ2VtZW50IGFuZCBwcmVzZXJ2YXRpb24gb2YgdGhlIGxha2VzIGF0ICBwcmVzZW50IGhhdmUgYmVjb21lIGEgZ3JlYXQgY2hhbGxlbmdlIGZvciBlbnZpcm9ubWVudGFsLWlzdHMgYW5kIG1hbmFnZXJzIFs1LCA5XS4gVGhlIGV1dHJvcGhpY2F0aW9uIGlzIGFsc28gYSBtYWluIHByb2JsZW0gb2YgdGhlICBmcmVzaHdhdGVyIGxha2VzLCBhbmQgaXQgaXMgc3BlY2lmaWVkIGZvciB0aGUgbGFrZXMgaGF2aW5nICBudXRyaWVudCBlbnJpY2htZW50LiBJdCBjYXVzZXMgZXhjZXNzaXZlIGdyb3d0aCBvZiBhbGdhZSAgYW5kIGlzIGNvbnNpZGVyZWQgdG8gYmUgcmVzcG9uc2libGUgZm9yIGRlZ3JhZGluZyB0aGUgIHF1YWxpdGF0aXZlIG5hdHVyZSBvZiB0aGUgbGFrZXMgWzUsIDEwXS4gVGhlIGV1dHJvcGhpY2F0aW9uICBpcyBmb2xsb3dlZCBieSB0aGUgYWxnYWwgYmxvb20sIHdoaWNoIGdlbmVyYXRlcyB0b3hpbiwgIHdvcnNlbnMgd2F0ZXIgcXVhbGl0eSwgZGVwbGV0ZXMgZGlzc29sdmVkIG94eWdlbiAoRE8pICogUGF3YW4gS3VtYXIsIHBhd2FuLmV2c0BnbWFpbC5jb20gfCAxRGVwYXJ0bWVudCBvZiBFbnZpcm9ubWVudGFsIFNjaWVuY2VzLCBDZW50cmFsIFVuaXZlcnNpdHkgb2YgSGltYWNoYWwgUHJhZGVzaCwgRGhhcmFtc2hhbGEgMTc2MjE1LCBJbmRpYS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2OC41Nzg0NjA2OTMzNTkzOCwibGVmdCI6OC41NzE0MTg3NjIyMDcwMzEsIndpZHRoIjo0MC4zMjc1MTQ2NDg0Mzc1LCJoZWlnaHQiOjEuNTE5ODU1MjYwODQ4OTk5fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NzAuMTU4OTk2NTgyMDMxMjUsImxlZnQiOjguNTY5NzM4Mzg4MDYxNTIzLCJ3aWR0aCI6NDAuMzU0NzI0ODg0MDMzMiwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjcxLjczOTU0NzcyOTQ5MjE5LCJsZWZ0Ijo4LjU2OTczODM4ODA2MTUyMywid2lkdGgiOjQwLjM0OTY5MzI5ODMzOTg0NCwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjczLjMyMDA5MTI0NzU1ODYsImxlZnQiOjguNTY4MDU5OTIxMjY0NjQ4LCJ3aWR0aCI6NDAuMDIwMDg4MTk1ODAwNzgsImhlaWdodCI6MS41MTk4NTUyNjA4NDg5OTl9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo3NC45MDA2NDIzOTUwMTk1MywibGVmdCI6OC41NjgwNTk5MjEyNjQ2NDgsIndpZHRoIjozOS45OTk5Mjc1MjA3NTE5NSwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjc2LjQ4MTE4NTkxMzA4NTk0LCJsZWZ0Ijo4LjU2ODA1NzA2MDI0MTcsIndpZHRoIjo0MC4zNTcwNzQ3Mzc1NDg4MywiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjc4LjA2MTcyOTQzMTE1MjM0LCJsZWZ0Ijo4LjU2OTczNjQ4MDcxMjg5LCJ3aWR0aCI6NDAuMzUzNzEzOTg5MjU3ODEsImhlaWdodCI6MS41MTk4NTUyNjA4NDg5OTl9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2MC42NzU3Mjc4NDQyMzgyOCwibGVmdCI6NTEuNDI3MTY1OTg1MTA3NDIsIndpZHRoIjo0MC4zNzg1NjY3NDE5NDMzNiwiaGVpZ2h0IjoxOC45MDU4NTUxNzg4MzMwMDh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo4MS45MjczMzc2NDY0ODQzOCwibGVmdCI6OC40MDE0OTc4NDA4ODEzNDgsIndpZHRoIjo3My45NTQ2OTY2NTUyNzM0NCwiaGVpZ2h0IjoyLjY3ODc2MjY3NDMzMTY2NX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEifX1d. Economic incentives amplify impact market-based strategies rewarding verified reports offset losses like INR 500-1000 crore in fisheries, mirroring U.S. models where volunteer data informs adaptive policies (Berdalet et al., 2017; Foulon et al., 2019)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IsOJdGllbm5lIiwiZmFtaWx5IjoiRm91bG9uIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFsYWluIE4uIiwiZmFtaWx5IjoiUm91c3NlYXUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiR2xlbm4iLCJmYW1pbHkiOiJCZW5veSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJSZWJlY2NhIEwuIiwiZmFtaWx5IjoiTm9ydGgifX1dLCJpZHMiOnsiaXNzbiI6WyIxMjAxLTMwODAiLCIyNDA4LTk0NDMiXSwiaXNibiI6W10sImRvaSI6IjEwLjIxNjYvd3Fyai4yMDE5LjAxMyIsIm1hZyI6IjI5NzM4NjAyODEiLCJvcGVuYWxleCI6IlcyOTczODYwMjgxIn0sInRpdGxlIjoiQSBnbG9iYWwgc2NhbiBvZiBob3cgdGhlIGlzc3VlIG9mIG51dHJpZW50IGxvYWRpbmcgYW5kIGhhcm1mdWwgYWxnYWwgYmxvb21zIGlzIGJlaW5nIGFkZHJlc3NlZCBieSBnb3Zlcm5tZW50cywgbm9uLWdvdmVybm1lbnRhbCBvcmdhbml6YXRpb25zLCBhbmQgdm9sdW50ZWVycyIsImFic3RyYWN0IjoiQWJzdHJhY3QgSGFybWZ1bCBhbGdhbCBibG9vbXMgKEhBQnMpIGluIGFxdWF0aWMgZWNvc3lzdGVtcyBhcmUgb2YgY29uY2VybiB3b3JsZHdpZGUuIFRoaXMgcmV2aWV3IGRlYWxzIHdpdGggaG93IGp1cmlzZGljdGlvbnMgYXJvdW5kIHRoZSB3b3JsZCBhcmUgYWRkcmVzc2luZyB0aGlzIHdhdGVyIHF1YWxpdHkgaXNzdWUgdG8gaW5mb3JtIHJlY29tbWVuZGF0aW9ucyByZWdhcmRpbmcgbnV0cmllbnQgbG9hZGluZyBhbmQgSEFCcyBpbiBNaXNzaXNxdW9pIEJheS1MYWtlIENoYW1wbGFpbiBhbmQgTGFrZSBNZW1waHJlbWFnb2c7IHRyYW5zYm91bmRhcnkgbGFrZXMgbG9jYXRlZCBpbiB0aGUgVVNBIGFuZCBDYW5hZGEgdGhhdCBzdWZmZXIgZnJvbSBzeW1wdG9tcyBvZiBldXRyb3BoaWNhdGlvbi4gQSBnbG9iYWwgc2NhbiBvZiB0aGUgbGl0ZXJhdHVyZSByZXN1bHRlZCBpbiB0aGUgY29uc2lkZXJhdGlvbiBvZiAxMiBjYXNlIHN0dWRpZXMgb2YgbGFyZ2Ugd2F0ZXIgYm9kaWVzIHdpdGhpbiBsYXJnZSB3YXRlcnNoZWRzLCBleGNsdWRpbmcgaW4tbGFrZSBnZW9lbmdpbmVlcmluZyBhcHByb2FjaGVzLiBBbHRob3VnaCBhbGwgb2YgdGhlIHN5c3RlbXMgZXhwZXJpZW5jZSBleGNlc3NpdmUgbnV0cmllbnQgbG9hZGluZywgdGhleSB2YXJ5IGluIHR3byBrZXkgd2F5czogc291cmNlcyBvZiBudXRyaWVudHMgYW5kIG1hbmlmZXN0YXRpb25zIG9mIGV1dHJvcGhpY2F0aW9uIHJhbmdpbmcgZnJvbSBIQUJzLCB0byBsaW1pdGVkIHJlY3JlYXRpb25hbCB1c2VzLCB0byB0aGUgYWRkaXRpb25hbCBjb21wbGV4aXR5IG9mIGludGVybmFsIGxvYWRpbmdzIGFuZCBmaXNoIGtpbGxzLCB1cCB0byBkcmlua2luZyB3YXRlciBzaHV0ZG93bnMuIFRoZSBjYXNlIHN0dWRpZXMgd2VyZSBhbmFseXplZCB3aXRoIHJlc3BlY3QgdG8gZm91ciBjYXRlZ29yaWVzIG9mIGFwcHJvYWNoZXMsIG5hbWVseTogKGkpIHJlZ3VsYXRvcnk7IChpaSkgaW5jZW50aXZlLWJhc2VkOyAoaWlpKSByaXNrIG1pdGlnYXRpb247IGFuZCAoaXYpIG91dHJlYWNoLCBlbmdhZ2VtZW50LCBhbmQgZWR1Y2F0aW9uYWwuIFdlIGZvdW5kIHRoYXQgdGhlIG1hbmFnZW1lbnQgZnJhbWV3b3JrcyBhcmUgYmFzZWQgb24gaW50ZWdyYXRlZCB3YXRlcnNoZWQgbWFuYWdlbWVudCBwbGFubmluZyBhbmQgbmF0aW9uYWwgc3RhbmRhcmRzLiBOYXRpb25hbCB3YXRlciBxdWFsaXR5IHN0YW5kYXJkcywgaG93ZXZlciwgYXJlIG5vdCBzdHJpbmdlbnQgZW5vdWdoIHRvIHByZXZlbnQgSEFCcy4gT3ZlcmFsbCwgaWRlbnRpZmllZCBjYXNlIHN0dWRpZXMgZGlkIG5vdCBzdWNjZXNzZnVsbHkgcmVtZWRpYXRlIEhBQnMsIHRoZXkgc2ltcGx5IG1hbmFnZWQgdGhlbS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjE2Ni93cXJqLjIwMTkuMDEzIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2l3YXBvbmxpbmUuY29tL3dxcmovYXJ0aWNsZS1wZGYvNTUvMS8xLzcwOTI2MS93cXJqYzA1NTAwMDEucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMTktMDktMTciLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA5IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJXYXRlciBRdWFsaXR5IFJlc2VhcmNoIEpvdXJuYWwiLCJpc3N1ZSI6MSwidm9sdW1lIjo1NSwiaW1wYWN0RmFjdG9yIjoiNy44MyJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjEsImxhc3RQYWdlIjoyM319LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJVV0EgUHVibGlzaGluZyJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6ODQsImNpdGVkQnlDb3VudCI6MjB9fSwicXVvdGUiOiJXZSBwcm9wb3NlIHNvbHV0aW9ucyBhbmQgcGF0aHMgZm9yd2FyZCB0byBhIHdpY2tlZCBwcm9ibGVtIHdoaWNoIHNob3VsZCBwcm92ZSBpbnNpZ2h0ZnVsIGZvciBwb2xpY3ktbWFrZXJzIGFuZCBtYW4tIGFnZXJzIGFsaWtlIHRocm91Z2ggdGhlIGZvbGxvd2luZyBmaW5kaW5ncy4gTmF0aW9uYWwgd2F0ZXIgcXVhbGl0eSBzdGFuZGFyZHMgaGF2ZSBub3QgcHJvdmVkIHN0cmluZ2VudCBlbm91Z2ggdG8gcHJlLSB2ZW50IEhBQnMuIElkZW50aWZpZWQgZnJhbWV3b3JrcyBhcmUgYWxsIGJhc2VkIG9uIGludGVncmF0ZWQgd2F0ZXJzaGVkIG1hbmFnZW1lbnQgYW5kIG5hdGlvbmFsIHN0YW5kYXJkcy4gTm8gYWN0dWFsIHN1Y2Nlc3Mgc3RvcmllcyByZWxhdGVkIHRvIHJlbWVkaWF0aW9uIG9mIEhBQnMgd2VyZSBpZGVudGlmaWVkOyB0aGV5IHdlcmUgc2ltcGx5IG1hbmFnZW1lbnQgc3Rvcmllcy4gVGhlIGVmZmljYWN5IG9mIE5QUyBCTVBzIGlzIHF1ZXN0aW9uZWQgdGhyb3VnaG91dCB0aGUgd29ybGQgZHVlIHRvIGluc3VmZmljaWVudCBldmlkZW5jZSB0byBkZXRlcm1pbmUgd2hldGhlciB0aGV5IGNhbiBtaXRpZ2F0ZSBIQUJzLiBNYXJrZXQtYmFzZWQgc3RyYXRlZ2llcyB0byBvZmZzZXQgcG9sbHV0aW9uIG9yIHByb21vdGUgdGFyZ2V0ZWQgQk1QIGFkb3B0aW9uIGFyZSBiZWNvbWluZyBwb3B1bGFyIGFuZCByZXN1bHRpbmcgaW4gY29zdC1lZmZlY3RpdmUgYW5kIGVmZmljaWVudCBhcHByb2FjaGVzLiBDb3N04oCTYmVuZWZpdCBhbmFseXNlcyBtYXkgaW5jcmVhc2UgdGhlIHVzZSBvZiB0ZWNobmljYWwgc29sLSB1dGlvbnMgb3ZlciBjb21wcmVoZW5zaXZlIGFwcHJvYWNoZXMuIEF0IHRoaXMgcG9pbnQsIHRoZXJlIGlzIG5vdCByZWFsbHkgYSB1bmlxdWUgY2FzZSBzdHVkeSB0aGF0IGNvdWxkIGJlIGFwcGxpZWQgdG8gTGFrZSBDaGFtcGxhaW4gYW5kIExha2UgTWVtcGhyZW1hZ29nLiBTcGVjaWZpYyBlbGVtZW50cyBvZiB0aGUgYXBwcm9hY2hlcyBpZGVudGlmaWVkIGluIHRoaXMgcmV2aWV3IGFuZCB1bmktIGZ5aW5nIHRoZW1lcywgaG93ZXZlciwgaG9sZCBwcm9taXNlcy4gVGhpcyBpcyB3aHkgd2UgYWR2b2NhdGUgZm9yIGNvbmR1Y3RpbmcgYSBzeXN0ZW1hdGljIGxpdGVyYXR1cmUgcmV2aWV3IHdpdGggcmVnYXJkIHRvIHRob3NlIHRoZW1lcyBpbiBvcmRlciB0byBkZXZlbG9wIHJlY29tbWVuLSBkYXRpb25zIHRhaWxvcmVkIHRvIHRoZSBMYWtlIE1lbXBocmVtYWdvZyBhbmQgTWlzc2lzcXVvaSBCYXktTGFrZSBDaGFtcGxhaW4gYXJlYXMuIEF0IGxhc3QsIHdlIHdvdWxkIGxpa2UgdG8gYWRkcmVzcyB0aGUgZWxlcGhhbnQtc2l6ZWQgcXVlcy0gdGlvbiBpbiB0aGUgcm9vbS4gV2lsbCBjbGltYXRlIGNoYW5nZSBpbmNyZWFzZSBIQUJzIGZyZXF1ZW5jeT8gSW50ZXJhY3RpdmUgZWZmZWN0cyBvZiBldXRyb3BoaWNhdGlvbiBhbmQgY2xpbWF0ZSBjaGFuZ2Ugb24gaGFybWZ1bCBjeWFub2JhY3RlcmlhbCBibG9vbXMgYXJlIGNvbXBsZXguICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxNSwiYmJveCI6eyJ0b3AiOjg1LjYwNTUxNDUyNjM2NzE5LCJsZWZ0Ijo0OS43OTQ4NzIyODM5MzU1NSwid2lkdGgiOjM5LjcxODY2MjI2MTk2Mjg5LCJoZWlnaHQiOjEuMzI4NjcxMzM2MTc0MDExMn19LHsicGFnZU51bWJlciI6MTUsImJib3giOnsidG9wIjo4Ny4zOTUzNjI4NTQwMDM5LCJsZWZ0Ijo0OS43OTQ4NzIyODM5MzU1NSwid2lkdGgiOjM5LjcwMjcwOTE5Nzk5ODA1LCJoZWlnaHQiOjEuMzI4NjcxMzM2MTc0MDExMn19LHsicGFnZU51bWJlciI6MTUsImJib3giOnsidG9wIjo4OS4xODUyMTExODE2NDA2MiwibGVmdCI6NDkuNzk0ODcyMjgzOTM1NTUsIndpZHRoIjozOS43MDIwOTUwMzE3MzgyOCwiaGVpZ2h0IjoxLjMyODY3MTMzNjE3NDAxMTJ9fSx7InBhZ2VOdW1iZXIiOjE1LCJiYm94Ijp7InRvcCI6OTAuOTc1MDU5NTA5Mjc3MzQsImxlZnQiOjQ5Ljc5NDg3MjI4MzkzNTU1LCJ3aWR0aCI6MzkuNzA0NjIwMzYxMzI4MTI1LCJoZWlnaHQiOjEuMzI4NjcxMzM2MTc0MDExMn19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjoxNC4wMjU5OTA0ODYxNDUwMiwibGVmdCI6MTAuNTAyMzIzMTUwNjM0NzY2LCJ3aWR0aCI6MzkuNzA1OTU1NTA1MzcxMDk0LCJoZWlnaHQiOjEuMzI4NjY5NDI4ODI1Mzc4NH19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjoxNS44MTU4NDE2NzQ4MDQ2ODgsImxlZnQiOjEwLjUwMjMyMzE1MDYzNDc2Niwid2lkdGgiOjM5LjcwMjI0NzYxOTYyODkwNiwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MTcuNjA1NjkxOTA5NzkwMDQsImxlZnQiOjEwLjUwMjE3NjI4NDc5MDAzOSwid2lkdGgiOjM5LjcwMDkyMzkxOTY3NzczNCwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MTkuMzk1NTQ0MDUyMTI0MDIzLCJsZWZ0IjoxMC41MDIxNzYyODQ3OTAwMzksIndpZHRoIjozOS43MDg5MDA0NTE2NjAxNTYsImhlaWdodCI6MS4zMjg2Njk0Mjg4MjUzNzg0fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjIxLjE4NTM5NDI4NzEwOTM3NSwibGVmdCI6MTAuNTAyMTc2Mjg0NzkwMDM5LCJ3aWR0aCI6MzkuNzEwMDgzMDA3ODEyNSwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MjIuOTc1MjQ0NTIyMDk0NzI3LCJsZWZ0IjoxMC41MDIxNzYyODQ3OTAwMzksIndpZHRoIjozOS43MDQwMjkwODMyNTE5NSwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MjQuNzY1MDk2NjY0NDI4NzEsImxlZnQiOjEwLjUwMjE3NjI4NDc5MDAzOSwid2lkdGgiOjM5LjcwMzg4MDMxMDA1ODU5NCwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MjYuNTU0OTQ4ODA2NzYyNjk1LCJsZWZ0IjoxMC41MDIwMjc1MTE1OTY2OCwid2lkdGgiOjM5LjcwMzg4MDMxMDA1ODU5NCwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MjguMzQ0Nzk5MDQxNzQ4MDQ3LCJsZWZ0IjoxMC41MDIwMjc1MTE1OTY2OCwid2lkdGgiOjM5LjcwMDAzODkwOTkxMjExLCJoZWlnaHQiOjEuMzI4NjY5NDI4ODI1Mzc4NH19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjozMC4xMzQ2NDczNjkzODQ3NjYsImxlZnQiOjEwLjUwMjAyNzUxMTU5NjY4LCJ3aWR0aCI6MzkuNzA0NjE2NTQ2NjMwODYsImhlaWdodCI6MS4zMjg2Njk0Mjg4MjUzNzg0fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjMxLjkyNDQ5NzYwNDM3MDExNywibGVmdCI6MTAuNTAxODc3Nzg0NzI5MDA0LCJ3aWR0aCI6MzkuNzA1Nzg3NjU4NjkxNDA2LCJoZWlnaHQiOjEuMzI4NjY5NDI4ODI1Mzc4NH19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjozMy43MTQzNTE2NTQwNTI3MzQsImxlZnQiOjEwLjUwMTcyOTAxMTUzNTY0NSwid2lkdGgiOjM5LjcxOTI0NTkxMDY0NDUzLCJoZWlnaHQiOjEuMzI4NjY3NDAyMjY3NDU2fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjM1LjUwNDIwMzc5NjM4NjcyLCJsZWZ0IjoxMC41MDE3MjkwMTE1MzU2NDUsIndpZHRoIjozOS43MDI3MDkxOTc5OTgwNSwiaGVpZ2h0IjoxLjMyODY2NzQwMjI2NzQ1Nn19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjozNy4yOTQwNTIxMjQwMjM0NCwibGVmdCI6MTAuNTAxNzI5MDExNTM1NjQ1LCJ3aWR0aCI6MzkuNzA1Nzk5MTAyNzgzMiwiaGVpZ2h0IjoxLjMyODY2NzQwMjI2NzQ1Nn19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjozOS4wODM5MDQyNjYzNTc0MiwibGVmdCI6MTAuNTAxNzI1MTk2ODM4Mzc5LCJ3aWR0aCI6MzkuNzA2NTM1MzM5MzU1NDcsImhlaWdodCI6MS4zMjg2Njc0MDIyNjc0NTZ9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6NDAuODczNzU2NDA4NjkxNDA2LCJsZWZ0IjoxMC41MDE3MjcxMDQxODcwMTIsIndpZHRoIjozOS43MjExNTMyNTkyNzczNDQsImhlaWdodCI6MS4zMjg2Njc0MDIyNjc0NTZ9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6NDIuNjYzNjA4NTUxMDI1MzksImxlZnQiOjEwLjUwMTcyNzEwNDE4NzAxMiwid2lkdGgiOjM5LjcwMzEzNjQ0NDA5MTgsImhlaWdodCI6MS4zMjg2Njc0MDIyNjc0NTZ9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6NDQuNDQ2MjA4OTUzODU3NDIsImxlZnQiOjEwLjUwMTcyNzEwNDE4NzAxMiwid2lkdGgiOjM5LjcwMDc3NTE0NjQ4NDM3NSwiaGVpZ2h0IjoxLjMyODY2NzQwMjI2NzQ1Nn19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjo0Ni4yMzYwNjEwOTYxOTE0MDYsImxlZnQiOjEwLjUwMTcyNzEwNDE4NzAxMiwid2lkdGgiOjM5LjcwNzcyNTUyNDkwMjM0NCwiaGVpZ2h0IjoxLjMyODY2NzQwMjI2NzQ1Nn19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjo0OC4wMjU5MTMyMzg1MjUzOSwibGVmdCI6MTAuNTAxNzI3MTA0MTg3MDEyLCJ3aWR0aCI6MjUuOTQxNjY3NTU2NzYyNjk1LCJoZWlnaHQiOjEuMzI4NjY3NDAyMjY3NDU2fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjQ5LjgxNTc2MTU2NjE2MjExLCJsZWZ0IjoxMy4xMjUwMTYyMTI0NjMzNzksIndpZHRoIjozNy4wODExODA1NzI1MDk3NjYsImhlaWdodCI6MS4zMjg2Njc0MDIyNjc0NTZ9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6NTEuNjA1NjEzNzA4NDk2MDk0LCJsZWZ0IjoxMC41MDE3MjcxMDQxODcwMTIsIndpZHRoIjozOS43MDkyMDU2Mjc0NDE0MDYsImhlaWdodCI6MS4zMjg2Njc0MDIyNjc0NTZ9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6NTMuMzk1NDYyMDM2MTMyODEsImxlZnQiOjEwLjUwMTcyNzEwNDE4NzAxMiwid2lkdGgiOjM5LjcxOTIyMzAyMjQ2MDk0LCJoZWlnaHQiOjEuMzI4NjY3NDAyMjY3NDU2fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjU1LjE4NTMxMDM2Mzc2OTUzLCJsZWZ0IjoxMC41MDE3MjcxMDQxODcwMTIsIndpZHRoIjozOS43MTA2NzgxMDA1ODU5NCwiaGVpZ2h0IjoxLjMyODY2NzQwMjI2NzQ1Nn19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjE2In19LHsib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkVsaXNhIiwiZmFtaWx5IjoiQmVyZGFsZXQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUmFwaGFlbCBNLiIsImZhbWlseSI6Ikt1ZGVsYSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJFZCIsImZhbWlseSI6IlVyYmFuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhlbnJpayIsImZhbWlseSI6IkVuZXZvbGRzZW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTmVpbCBTLiIsImZhbWlseSI6IkJhbmFzIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkVpbGVlbiIsImZhbWlseSI6IkJyZXNuYW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTWljaGVsZSBBLiIsImZhbWlseSI6IkJ1cmZvcmQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiS2VpdGgiLCJmYW1pbHkiOiJEYXZpZHNvbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJDaHJpc3RvcGhlciBKLiIsImZhbWlseSI6IkdvYmxlciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJCZW5ndCIsImZhbWlseSI6Ikthcmxzb24ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUG8gVGVlbiIsImZhbWlseSI6IkxpbSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJMaW5jb2xuIiwiZmFtaWx5IjoiTWFjS2VuemllIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1hcmluYSIsImZhbWlseSI6Ik1vbnRyZXNvciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJWZXJhIEwuIiwiZmFtaWx5IjoiVHJhaW5lciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJHaXJlcyIsImZhbWlseSI6IlVzdXAifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiS2Vkb25nIiwiZmFtaWx5IjoiWWluIn19XSwiaWRzIjp7Imlzc24iOlsiMTA0Mi04Mjc1IiwiMjM3Ny02MTdYIl0sImlzYm4iOltdLCJkb2kiOiIxMC41NjcwL29jZWFub2cuMjAxNy4xMTEiLCJtYWciOiIyNTk3NjY0NzI3Iiwib3BlbmFsZXgiOiJXMjU5NzY2NDcyNyJ9LCJ0aXRsZSI6Ikdsb2JhbEhBQjogQSBOZXcgUHJvZ3JhbSB0byBQcm9tb3RlIEludGVybmF0aW9uYWwgUmVzZWFyY2gsIE9ic2VydmF0aW9ucywgYW5kIE1vZGVsaW5nIG9mIEhhcm1mdWwgQWxnYWwgQmxvb21zIGluIEFxdWF0aWMgU3lzdGVtcyIsImFic3RyYWN0IjoiQmVyZGFsZXQsIEVsaXNhIC4uLiBldCBhbC4tLSBTcGVjaWFsIGlzc3VlIG9uIEludGVybmF0aW9uYWwgQ29vcGVyYXRpb24gaW4gSGFybWZ1bCBBbGdhbCBCbG9vbSBTY2llbmNlLi0tIDEyIHBhZ2VzLCA2IGZpZ3VyZXMiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuNTY3MC9vY2Vhbm9nLjIwMTcuMTExIiwicmVzb3VyY2VVcmwiOiJodHRwczovL3Rvcy5vcmcvb2NlYW5vZ3JhcGh5L2Fzc2V0cy9kb2NzLzMwLTFfYmVyZGFsZXQzLnBkZiIsImlzc3VlZERhdGUiOiIyMDE3LTAzLTAxIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiT2NlYW5vZ3JhcGh5IiwiaXNzdWUiOjEsInZvbHVtZSI6MzAsImltcGFjdEZhY3RvciI6IjIuOTMifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjo3MCwibGFzdFBhZ2UiOjgxfX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6Ik9jZWFub2dyYXBoeSBTb2NpZXR5In0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjoyNiwiY2l0ZWRCeUNvdW50IjozMn19LCJxdW90ZSI6IlRoZSAgZWNvbm9taWMgZWZmZWN0cyBvZiBIQUJzIGFyaXNlIGZyb20gcHViLWxpYyBoZWFsdGggY29zdHMsIGNvbW1lcmNpYWwgZmlzaGVyeSBhbmQgIGFxdWFjdWx0dXJlIG9wZXJhdGlvbiBjbG9zdXJlcyBhbmQgZmlzaCAga2lsbHMsIHBvc3NpYmxlIG1lZGl1bS0gYW5kIGxvbmctdGVybSAgZGVjbGluZXMgaW4gY29hc3RhbCBhbmQgbWFyaW5lIHJlY3JlYXRpb24gIGFuZCB0b3VyaXNtLCBhbmQgdGhlIGNvc3RzIG9mIGluc3VyYW5jZSwgIG1vbml0b3JpbmcsIG1hbmFnZW1lbnQsIGFuZCBtaXRpZ2F0aW9uICAoTW9yZ2FuIGV0IGFsLiwgMjAxMCkuIEFnZ3JlZ2F0aW5nIGVjby1ub21pYyBlZmZlY3Rz4oCUYm90aCB3aXRoaW4gYW5kIGFjcm9zcyAgdGhlc2UgY2F0ZWdvcmllc%2BKAlGNhbiBhbHNvIGJlIHByb2JsZW0tYXRpYyB3aXRoIGF2YWlsYWJsZSBkYXRhIGFuZCBjdXJyZW50bHkgdXNlZCAgbWV0aG9kb2xvZ2llcyAoSG9hZ2xhbmQgZXQgYWwuLCAyMDAyKS4gIEFuIGV4YW1wbGUgb2YgYW4gZWNvbm9taWMgZXZhbHVhdGlvbiAgb2YgSEFCcyBhdCBhIG5hdGlvbmFsIGxldmVsIGlzIGdpdmVuIGJ5ICBBbmRlcnNvbiBldCBhbC4gKDIwMDApLCB3aG8gZXN0aW1hdGVkIHRoZSAgZWNvbm9taWMgZWZmZWN0cyBvZiBIQUJzIGluIHRoZSBVbml0ZWQgIFN0YXRlcyB0byBiZSAkMTAwIG1pbGxpb24gcGVyIHllYXIgKGFzIGxhdGVyICB0cmFuc2xhdGVkIGludG8gMjAxMiBkb2xsYXJzKSwgYnJva2VuIGRvd24gIGFzIGZvbGxvd3M6IDQ1JSBmb3IgcHVibGljIGhlYWx0aCBjb3N0cywgIDM3JSBmb3IgdGhlIGNvc3RzIG9mIGNsb3N1cmVzIGFuZCBsb3NzZXMgIGV4cGVyaWVuY2VkIGJ5IGNvbW1lcmNpYWwgZmlzaGVyaWVzLCAgMTMlIGZyb20gdGhlIGltcGFjdCBvbiBsb3N0IHJlY3JlYXRpb24gIGFuZCB0b3VyaXNtIG9wcG9ydHVuaXRpZXMsIGFuZCA0JSBmcm9tIEZJR1VSRSA2LiBNYXNzaXZlIGZpc2ggbW9ydGFsaXRpZXMgaW4gSmFrYXJ0YSwgSW5kb25lc2lhLiBQaG90byBjcmVkaXRzOiBUdW1wYWsgU2lkYWJ1dGFyIChIQU41MykgbW9uaXRvcmluZyBhbmQgbWFuYWdlbWVudCBjb3N0cy4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjoxMS44MTk0NDQ2NTYzNzIwNywibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzQ5NDUyOTcyNDEyMSwiaGVpZ2h0IjoxLjcwMzI4MjU5NDY4MDc4NjF9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjoxMy41MjM5ODk2Nzc0MjkyLCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI2Ljg2NDM3NDE2MDc2NjYsImhlaWdodCI6MS43MDMyODI1OTQ2ODA3ODYxfX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6MTUuMjI4NTM1NjUyMTYwNjQ1LCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzMjAwMjI1ODMwMDc4LCJoZWlnaHQiOjEuNzAzMjgzNTQ4MzU1MTAyNX19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjE2LjkzMzA4MDY3MzIxNzc3MywibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzM2MzY4NTYwNzkxLCJoZWlnaHQiOjEuNzAzMjgzNTQ4MzU1MTAyNX19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjE4LjYzNzYyNjY0Nzk0OTIyLCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzNTI5NDM0MjA0MTAxNiwiaGVpZ2h0IjoxLjcwMzI4MzU0ODM1NTEwMjV9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjoyMC4zNDIxNzA3MTUzMzIwMywibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzM5ODU5MDA4Nzg5MDYsImhlaWdodCI6MS43MDMyODM1NDgzNTUxMDI1fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6MjIuMDQ2NzE2NjkwMDYzNDc3LCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzMjAxMTc5NTA0Mzk0NSwiaGVpZ2h0IjoxLjcwMzI4MzU0ODM1NTEwMjV9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjoyMy43NTEyNjI2NjQ3OTQ5MjIsImxlZnQiOjY3LjI1NDg5ODA3MTI4OTA2LCJ3aWR0aCI6MjcuMjMyMDQyMzEyNjIyMDcsImhlaWdodCI6MS43MDMyODM1NDgzNTUxMDI1fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6MjUuNDU1ODA4NjM5NTI2MzY3LCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI2Ljg2MzA2NzYyNjk1MzEyNSwiaGVpZ2h0IjoxLjcwMzI4MzU0ODM1NTEwMjV9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjoyNy4xNjAzNTI3MDY5MDkxOCwibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzU0MzM1Nzg0OTEyMSwiaGVpZ2h0IjoxLjcwMzI4MzU0ODM1NTEwMjV9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjoyOC44NjQ4OTg2ODE2NDA2MjUsImxlZnQiOjY3LjI1NDg5ODA3MTI4OTA2LCJ3aWR0aCI6MjYuODY0Mzk1MTQxNjAxNTYyLCJoZWlnaHQiOjEuNzAzMjgzNTQ4MzU1MTAyNX19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjMwLjU2OTQ0Mjc0OTAyMzQzOCwibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzQ5MjQzMTY0MDYyNSwiaGVpZ2h0IjoxLjcwMzI4MzU0ODM1NTEwMjV9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjozMi4yNzM5OTA2MzExMDM1MTYsImxlZnQiOjY3LjI1NDg5ODA3MTI4OTA2LCJ3aWR0aCI6MjcuMjM0Nzc1NTQzMjEyODksImhlaWdodCI6MS43MDMyODM1NDgzNTUxMDI1fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6MzMuOTc4NTM0Njk4NDg2MzMsImxlZnQiOjY3LjI1NDg5ODA3MTI4OTA2LCJ3aWR0aCI6MjcuMjM0MzM2ODUzMDI3MzQ0LCJoZWlnaHQiOjEuNzAzMjg1NDU1NzAzNzM1NH19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjM1LjY4MzA3ODc2NTg2OTE0LCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzMjE2MDU2ODIzNzMwNSwiaGVpZ2h0IjoxLjcwMzI4NTQ1NTcwMzczNTR9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjozNy4zODc2MjI4MzMyNTE5NSwibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzM2NjU0NjYzMDg1OTQsImhlaWdodCI6MS43MDMyODU0NTU3MDM3MzU0fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6MzkuMDkyMTcwNzE1MzMyMDMsImxlZnQiOjY3LjI1NDg5ODA3MTI4OTA2LCJ3aWR0aCI6MjcuMjM2NTc5ODk1MDE5NTMsImhlaWdodCI6MS43MDMyODU0NTU3MDM3MzU0fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6NDAuNzk2NzE0NzgyNzE0ODQ0LCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzNjcxNzIyNDEyMTA5NCwiaGVpZ2h0IjoxLjcwMzI4NTQ1NTcwMzczNTR9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo0Mi41MDEyNjI2NjQ3OTQ5MiwibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzM2MjczMTkzMzU5MzgsImhlaWdodCI6MS43MDMyODU0NTU3MDM3MzU0fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6NDQuMjA1ODA2NzMyMTc3NzM0LCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzMDU0NTA0Mzk0NTMxMiwiaGVpZ2h0IjoxLjcwMzI4NTQ1NTcwMzczNTR9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo0NS45MTAzNTA3OTk1NjA1NSwibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzg1NDQ0NjQxMTEzMjgsImhlaWdodCI6MS43MDMyODU0NTU3MDM3MzU0fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6NDcuNjE0ODk0ODY2OTQzMzYsImxlZnQiOjY3LjI1NDg5ODA3MTI4OTA2LCJ3aWR0aCI6MjcuMjM0MTQ2MTE4MTY0MDYyLCJoZWlnaHQiOjEuNzAzMjg1NDU1NzAzNzM1NH19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjQ5LjMxOTQ0Mjc0OTAyMzQ0LCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzNTk0MDkzMzIyNzU0LCJoZWlnaHQiOjEuNzAzMjg1NDU1NzAzNzM1NH19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjUxLjAyMzk5MDYzMTEwMzUxNiwibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzY5MDc5NTg5ODQzNzUsImhlaWdodCI6MS43MDMyODU0NTU3MDM3MzU0fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6OTIuMjc4NzcwNDQ2Nzc3MzQsImxlZnQiOjguODIzNTI5MjQzNDY5MjM4LCJ3aWR0aCI6NTYuMjYzOTE2MDE1NjI1LCJoZWlnaHQiOjEuMjA5MDg5NTE3NTkzMzgzOH19LHsicGFnZU51bWJlciI6OSwiYmJveCI6eyJ0b3AiOjUuMDAxMjYyNjY0Nzk0OTIyLCJsZWZ0Ijo1Ljg4MjM1MjgyODk3OTQ5Miwid2lkdGgiOjI3LjIzNTI5NDM0MjA0MTAxNiwiaGVpZ2h0IjoxLjcwMzI4MjU5NDY4MDc4NjF9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI3OCJ9fV0%3D. Moreover, integrating citizen observations with satellite imagery detects spatial bloom patterns, as in Loktak Lake, where phumdi mats exacerbate hypoxia (Nõges et al., 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRpaW5hIiwiZmFtaWx5IjoiTsO1Z2VzIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hc2ltZSIsImZhbWlseSI6IkphbmF0aWFuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlJlZXQiLCJmYW1pbHkiOiJMYXVnYXN0ZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQZWV0ZXIiLCJmYW1pbHkiOiJOw7VnZXMifX1dLCJpZHMiOnsiaXNzbiI6WyIyMzUxLTk4OTQiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMTYvai5nZWNjby4yMDIwLmUwMTM2OSIsIm1hZyI6IjMxMDU0NDYwOTAiLCJvcGVuYWxleCI6IlczMTA1NDQ2MDkwIiwiZXhhIjoiaHR0cHM6Ly93d3cuc2NpZW5jZWRpcmVjdC5jb20vc2NpZW5jZS9hcnRpY2xlL3BpaS9TMjM1MTk4OTQyMDMwOTEwMCJ9LCJ0aXRsZSI6IlBvc3Qtc292aWV0IGNoYW5nZXMgaW4gbml0cm9nZW4gYW5kIHBob3NwaG9ydXMgc3RvaWNoaW9tZXRyeSBpbiB0d28gbGFyZ2Ugbm9uLXN0cmF0aWZpZWQgbGFrZXMgYW5kIHRoZSBpbXBhY3Qgb24gcGh5dG9wbGFua3RvbiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2ouZ2VjY28uMjAyMC5lMDEzNjkiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2ouZ2VjY28uMjAyMC5lMDEzNjkiLCJpc3N1ZWREYXRlIjoiMjAyMC0xMS0yMCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTEiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6Ikdsb2JhbCBFY29sb2d5IGFuZCBDb25zZXJ2YXRpb24iLCJ2b2x1bWUiOjI0LCJpbXBhY3RGYWN0b3IiOiIzLjU5In0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkVsc2V2aWVyIEJWIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo3MSwiY2l0ZWRCeUNvdW50IjoxNH19LCJxdW90ZSI6IlJhbmRvbSBGb3Jlc3QgYW5hbHlzaXMgaW5kaWNhdGVkIGFuIGltcG9ydGFudCByb2xlIG9mIGxpZ2h0IGxpbWl0YXRpb24gaW4gYm90aCBsYWtlcy4gQ29uc3RhbnRseSBsb3dlciBsZXZlbHMgb2YgTiBpbiB0aGUgZGVlcGVyIExha2UgUGVpcHNpIGZhdm91cmVkIE4yLWZpeGluZyBzcGVjaWVzLCB3aGljaCwgYXMgYSBwYXJhZG94LCBiZWNhbWUgUC1saW1pdGVkLiBBcyBjbGltYXRlIHdhcm1pbmcgcmVpbmZvcmNlcyBldXRyb3BoaWNhdGlvbiBwaGVub21lbmEgaW4gbGFrZXMgYnkgaW5jcmVhc2luZyBpbnRlcm5hbCBudXRyaWVudCBsb2FkaW5nIGFuZCBmYXZvdXJpbmcgYmxvb20tZm9ybWluZyBjeWFub2JhY3RlcmlhLCBtb3JlIHN0cmluZ2VudCBtZWFzdXJlcyB3b3VsZCBiZSBuZWVkZWQgdG8gZnVydGhlciBsaW1pdCBudXRyaWVudCBsb2FkcyAoZXNwZWNpYWxseSB0aGF0IG9mIFApIHRvIGxha2VzIHRocm91Z2ggaW1wcm92ZWQgd2FzdGV3YXRlciB0cmVhdG1lbnQgYW5kIGluY3JlYXNlZCBlZmZpY2llbmN5IG9mIGZlcnRpbGlzZXIgYXBwbGljYXRpb24uIExvbmctdGVybSBjaGFuZ2VzIGluIG51dHJpZW50IGxvYWRpbmcgYW5kIHN0b2ljaGlvbWV0cnkgaW4gd2F0ZXIgYm9kaWVzIHJlZmxlY3QgY2hhbmdlcyBpbiBhZ3JpY3VsdHVyYWwgcHJhY3RpY2VzIGFuZCB3YXN0ZXdhdGVyIHRyZWF0bWVudC4ifV0%3D. Challenges persist, including data validation amid corruption risks and exclusion of marginalized groups, yet hybrid platforms mitigate these by crowdsourcing verifiable uploads (Ji et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkRhb2JpbiIsImZhbWlseSI6IkppIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllhbnhpbmciLCJmYW1pbHkiOiJIYW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTGlhbmdob25nIiwiZmFtaWx5IjoiTG9uZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJYaWFva2FuZyIsImZhbWlseSI6IlhpbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJIdWl5YW4iLCJmYW1pbHkiOiJYdSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTaXFpYW4iLCJmYW1pbHkiOiJRaXUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSmlhbmdodWFpIiwiZmFtaWx5IjoiTWVuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJYaW5neGluZyIsImZhbWlseSI6IlpoYW8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiWWFuYW4iLCJmYW1pbHkiOiJIdWFuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJEZWZ1IiwiZmFtaWx5IjoiTGl1In19XSwiaWRzIjp7Imlzc24iOlsiMjI5Ni03MDFYIl0sImlzYm4iOltdLCJkb2kiOiIxMC4zMzg5L2Zldm8uMjAyMy4xMjk3MDQ3Iiwib3BlbmFsZXgiOiJXNDM5MDYxMDY2OCJ9LCJ0aXRsZSI6Ikh5cG94aWEgYW5kIGl0cyBmZWVkYmFjayByZXNwb25zZSB0byBhbGdhbCBibG9vbXMgYW5kIENINCBlbWlzc2lvbnMgaW4gc3VidHJvcGljYWwgcmVzZXJ2b2lycyIsImFic3RyYWN0IjoiQW50aHJvcG9nZW5pYyBudXRyaWVudCBpbnB1dCBzdGltdWxhdGVzIHN1cmZhY2UgZXV0cm9waGljYXRpb24gYW5kIGFsZ2FsIGJsb29tcyBhbmQgdGh1cyBleGFjZXJiYXRlcyBveHlnZW4gZGVwbGV0aW9uIGFuZCBoeXBveGlhIGluIGFxdWF0aWMgZWNvc3lzdGVtcy4gSG93ZXZlciwgZXhpc3Rpbmcgc3R1ZGllcyBoYXZlIHBhaWQgbGl0dGxlIGF0dGVudGlvbiBvbiB0aGUgdW5kZXJzdGFuZGluZyBvZiB0aGUgZmVlZGJhY2sgcmVsYXRpb25zaGlwcyBhbW9uZyBveHlnZW4gY29uc3VtcHRpb24sIGFsZ2FsIHN1Y2Nlc3Npb24gYW5kIGdyZWVuaG91c2UgZ2FzIGVmZmVjdCBhZnRlciByaXZlciBkYW1taW5nLCBwYXJ0aWN1bGFybHkgaW4gVGhyZWUgR29yZ2VzIFJlc2Vydm9pci4gVGhpcyBmaWVsZCB3b3JrIGludmVzdGlnYXRlZCB0aGUgdGhlcm1hbCBzdHJhdGlmaWNhdGlvbiwgd2F0ZXIgcXVhbGl0eSBhbmQgcGh5dG9wbGFua3RvbiBpbiBmb3VyIHRyaWJ1dGFyaWVzIChQZW5neGkgQmF5IChQWEIpLCBEYW5pbmcgQmF5IChETkIpLCBTaGVubm9uZyBCYXkgKFNOQiksIGFuZCBYaWFuZ3hpIEJheSAoWFhCKSkgb2YgdGhlIFRocmVlIEdvcmdlcyBSZXNlcnZvaXIgKFRHUikgaW4gc3ByaW5nIChBcHJpbCAyNS0yOCwgMjAyMiksIGFuZCBleHBsb3JlZCB0aGUgZHJpdmVycyBvZiBoeXBveGlhIGFuZCBpdHMgZmVlZGJhY2sgcmVzcG9uc2UgdG8gYWxnYWwgYmxvb21zIGFuZCBtZXRoYW5lIChDSCA0ICkgZW1pc3Npb25zLiBUaGUgcmVzdWx0cyBzaG93ZWQgdGhhdCBkdXJpbmcgdGhlIG9ic2VydmF0aW9uIHBlcmlvZCwgRE8sIHRoZXJtYWwgc3RyYXRpZmljYXRpb24sIGFuZCBhbGdhbCBibG9vbXMgd2VyZSBtb3JlIHNldmVyZSBpbiBQWEIuIFRoZSBoaWdoIENoZW1pY2FsIE94eWdlbiBEZW1hbmQgKENPRCBNbiApICgxMS4yNyBtZ%2BKAokwgLTEgKSBhbmQgRGlzc29sdmVkIE9yZ2FuaWMgQ2FyYm9uIChET0MpIGNvbnRlbnQgKDcuNTYgbWfigKJMIC0xICkgaW4gUFhCIHdlcmUgdGhlIG1haW4gY2F1c2VzIG9mIHRoZSBwcm9ncmVzc2l2ZSBoeXBveGlhLiBGdXJ0aGVybW9yZSwgc3Ryb25nZXIgZGVuc2l0eSBzdHJhdGlmaWNhdGlvbiBpbiBQWEIgaW5oaWJpdGVkIHRoZSB2ZXJ0aWNhbCBzdXBwbHkgb2YgRE8gYW5kIGFjY2VsZXJhdGVkIGJvdHRvbSBoeXBveGlhLiBCdXQgdGhlIGRvd25zbG9wZSBkZW5zaXR5IGN1cnJlbnQgcmVwbGVuaXNoZWQgYm90dG9tIERPIGluIHRoZSB1cHBlciByZWFjaGVzIG9mIHRoZSBETkIsIFNOQiwgYW5kIFhYQi4gVGhlIENINCBjb25jZW50cmF0aW9uIHdhcyBnZW5lcmFsbHkgaGlnaGVyIGluIGh5cG94aWEgem9uZSwgZXNwZWNpYWxseSBpbiB3aGVyZSBjeWFub2JhY3RlcmlhIGFyZSB0aGUgZG9taW5hbnQgc3BlY2llcyAoZWcuLCBQWDAzIGFuZCBQWDA0KS4gRGFtIGNvbnN0cnVjdGlvbiBsZWQgdG8gdGhlIHN1Y2Nlc3Npb24gb2YgYWxnYWwtZG9taW5hbnQgc3BlY2llcywgZXhlcnRpbmcgYSBzaWduaWZpY2FudCBpbmZsdWVuY2Ugb24gdGhlIHJpdmVyIGNhcmJvbiBjeWNsZS4gVGhlIGV4cGFuc2lvbiBvZiBjeWFub2JhY3RlcmlhbCBibG9vbXMgYW5kIHRoZSBjb25zZXF1ZW5jZSBvZiBtZXRhYm9saWMgc3Vic3RyYXRlcyBpbmNyZWFzZSBhcmUgZ3JhZHVhbGx5IGluY3JlYXNpbmcgb3h5Z2VuIGRlcHJpdmF0aW9uLCBldmVuIGh5cG94aWEsIGFuZCBlbmhhbmNpbmcgQ0ggNCBwcm9kdWN0aW9uIGluIHJpdmVycy4gVGhlc2UgZmluZGluZ3Mgc3VnZ2VzdCB0aGF0IHRoZSBzdWNjZXNzaW9uIG9mIGFsZ2FsLWRvbWluYW50IHNwZWNpZXMgY2F1c2VkIGJ5IHJpdmVyIGRhbW1pbmcgaGFzIGEgcHJvbm91bmNlZCBwb3NpdGl2ZSBmZWVkYmFjayBlZmZlY3Qgb24gcmVzZXJ2b2lyIGh5cG94aWEgcHJvY2Vzc2VzIGFuZCBncmVlbmhvdXNlIGVmZmVjdC4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMzM4OS9mZXZvLjIwMjMuMTI5NzA0NyIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cuZnJvbnRpZXJzaW4ub3JnL2FydGljbGVzLzEwLjMzODkvZmV2by4yMDIzLjEyOTcwNDcvcGRmP2lzUHVibGlzaGVkVjI9RmFsc2UiLCJpc3N1ZWREYXRlIjoiMjAyNC0wMS0wNSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZyb250aWVycyBpbiBFY29sb2d5IGFuZCBFdm9sdXRpb24iLCJ2b2x1bWUiOjExLCJpbXBhY3RGYWN0b3IiOiI0Ljc4In0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkZyb250aWVycyBNZWRpYSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NDAsImNpdGVkQnlDb3VudCI6NX19LCJxdW90ZSI6Im9nZW5pYyBudXRyaWVudCBpbnB1dCBzdGltdWxhdGVzIHN1cmZhY2UgZXV0cm9waGljYXRpb24gYW5kIGFsZ2FsIGJsb29tcyBhbmQgdGh1cyBleGFjZXJiYXRlcyBveHlnZW4gZGVwbGV0aW9uIGFuZCBoeXBveGlhIGluIGFxdWF0aWMgZWNvc3lzdGVtcy4gSG93ZXZlciwgZXhpc3Rpbmcgc3R1ZGllcyBoYXZlIHBhaWQgbGl0dGxlIGF0dGVudGlvbiBvbiB0aGUgdW5kZXJzdGFuZGluZyBvZiB0aGUgZmVlZGJhY2sgcmVsYXRpb25zaGlwcyBhbW9uZyBveHlnZW4gY29uc3VtcHRpb24sIGFsZ2FsIHN1Y2Nlc3Npb24gYW5kIGdyZWVuaG91c2UgZ2FzIGVmZmVjdCBhZnRlciByaXZlciBkYW1taW5nLCAuLi4gRE8gcGxheXMgYSBrZXkgcm9sZSBpbiB0aGUgYmlvZ2VvY2hlbWljYWwgY3ljbGUgb2Ygc3VyZmFjZSB3YXRlcnMgYW5kIGlzIGEgc2Vuc2l0aXZlIGluZGljYXRvciBvZiBwaHlzaWNhbCBhbmQgYmlvZ2VvY2hlbWljYWwgY2hhbmdlcyBpbiByZXNlcnZvaXIgYXF1YXRpYyBlY29zeXN0ZW1zICggV2V0emVsLCAyMDAxICkuIFdoZW4gd2F0ZXJzIGVudGVyIGFuIGFuYWVyb2JpYyBzdGF0ZSwgYXF1YXRpYyBvcmdhbmlzbXMgZGllIGluIGxhcmdlIG51bWJlcnMgYW5kIHRoZSByZWxlYXNlIG9mIHNlZGltZW50LXJlZHVjaW5nIHBvbGx1dGFudHMgKGUuZy4sIGFtbW9uaWEgbml0cm9nZW4sIC4uLiAyMDIyXSApLCBmdXJ0aGVyIGRldGVyaW9yYXRpbmcgd2F0ZXIgcXVhbGl0eS4gV2l0aCB0aGUgaW5jcmVhc2Ugb2YgcG9sbHV0aW9uIGxvYWRzIGluIHdhdGVyc2hlZHMgYW5kIHRoZSBpbnRlbnNpZmljYXRpb24gb2Ygd2F0ZXIgZXV0cm9waGljYXRpb24sIHdhdGVyIGh5cG94aWEgaGFzIGJlY29tZSBhIHNldmVyZSBnbG9iYWwgZWNvbG9naWNhbCBhbmQgZW52aXJvbm1lbnRhbCBwcm9ibGVtIGluIHJlc2Vydm9pcnMgKCBTdW4gZXTCoGFsLiwgMjAyMSA7IEphaXN3YWwgZXTCoGFsLiwgMjAyMyApLiBGb2N1c2luZyBvbiB0aGUgZHluYW1pYyBjaGFuZ2VzIGluIn1d. Platforms such as DataStream exemplify this infrastructure by curating and sharing community-gathered water quality data with the public (Simon et al., 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkRhbmEgRi4iLCJmYW1pbHkiOiJTaW1vbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJHYWJyaWVsIiwiZmFtaWx5IjoiTXVub3oifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUXVvYyBUdWMiLCJmYW1pbHkiOiJEaW5oIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlN1bmcgVm8iLCJmYW1pbHkiOiJEdXkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiS2F0IiwiZmFtaWx5IjoiS2F2YW5hZ2gifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUm9iZXJ0IEouIiwiZmFtaWx5IjoiU21pdGgifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQmFycnkiLCJmYW1pbHkiOiJIdXNrIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlPDqWJhc3RpZW4iLCJmYW1pbHkiOiJTYXV2w6kifX1dLCJpZHMiOnsiaXNzbiI6WyIyMDU3LTQ5OTEiXSwiaXNibiI6W10sImRvaSI6IjEwLjUzMzQvY3N0cC42NTUiLCJvcGVuYWxleCI6Ilc0MzkwNDA1NzM0Iiwib3BlbkFjY2Vzc0lkIjoiMTgxNzM5MDI1MDczNjI3NDcwNyJ9LCJ0aXRsZSI6IkFkb3B0IGEgTGFrZTogU3VjY2Vzc2Z1bGx5IFRyYWNraW5nIEhhcm1mdWwgQ3lhbm9iYWN0ZXJpYWwgQmxvb21zIGluIENhbmFkaWFuIFN1cmZhY2UgV2F0ZXJzIFRocm91Z2ggQ2l0aXplbiBTY2llbmNlIiwiYWJzdHJhY3QiOiJUaGUgcHJvbGlmZXJhdGlvbiBvZiBoYXJtZnVsIHdhdGVyYm9ybmUgY3lhbm9iYWN0ZXJpYWwgYWxnYWwgYmxvb21zLCBzb21lIG9mIHdoaWNoIGNhbiBwcm9kdWNlIHBvdGVudCB0b3hpbnMsIHBvc2VzIHNldmVyZSByaXNrcyB0byBlbnZpcm9ubWVudGFsIGFuZCBodW1hbiBoZWFsdGguIEFjYWRlbWljIGFuZCBnb3Zlcm5tZW50YWwgbW9uaXRvcmluZyBlZmZvcnRzIG1heSBiZSBjb25zdHJhaW5lZCBieSBidWRnZXQsIHRpbWUsIGFuZCBzdGFmZiwgYW5kIHRodXMgbWlzcyBvdGhlcndpc2Ugc2lnbmlmaWNhbnQgcG9sbHV0aW9uIGV2ZW50cy4gSGVyZSwgd2UgcmVwb3J0IG9uIHRoZSBpbXBsZW1lbnRhdGlvbiBvZiBhIGNpdGl6ZW4gc2NpZW5jZSBwcm9qZWN0IHRvIHRyYWNrIGhhcm1mdWwgY3lhbm9iYWN0ZXJpYWwgYmxvb21zIGluIGxha2VzIGFuZCB3YXRlcndheXMgYWNyb3NzIENhbmFkYS4gVGhyb3VnaCBib3RoIGNyb3dkc291cmNpbmcgYW5kIGNyb3dkZnVuZGluZywgdGhlIEFkb3B0IGEgTGFrZSAoQWRvcHQgYSBMYWtlIDIwMjIpIGNhbXBhaWduIGFpbWVkIHRvIGRvY3VtZW50IHRoZSBwb3RlbnRpYWwgcHJlc2VuY2Ugb2YgY3lhbm9iYWN0ZXJpYSBhbmQgdG94aW5zIHdpdGggdGhlIGFzc2lzdGFuY2Ugb2YgcGFydGljaXBhbnRzLCB0aHVzIGltcHJvdmluZyBwdWJsaWMgYXdhcmVuZXNzIG9mIHRoZSBpc3N1ZSBvZiB3YXRlciBxdWFsaXR5IHByZXNlcnZhdGlvbi4gVXNpbmcgc29jaWFsIG1lZGlhLCBwYXJ0aWNpcGFudHMgd2VyZSBlbmNvdXJhZ2VkIHRvIHBhcnRpY2lwYXRlIGluIHRoZSBpbml0aWF0aXZlIGJ5IGNvbGxlY3Rpbmcgc2FtcGxlcyBkdXJpbmcgYSBibG9vbSBmcm9tIGEgbmVhcmJ5IHBvbmQgb3IgYnkgbWFraW5nIGEgZmluYW5jaWFsIGNvbnRyaWJ1dGlvbiB0byBzdXBwb3J0IHRoZSBpbml0aWF0aXZlLiBBZG9wdCBhIExha2UgYmVuZWZpdHRlZCBmcm9tIHRoZSBhbmFseXRpY2FsIHBsYXRmb3JtIG9mIEFsZ2FsIEJsb29tcyBUcmVhdG1lbnQsIFJpc2sgQXNzZXNzbWVudCwgUHJlZGljdGlvbnMsIGFuZCBQcmV2ZW50aW9uIChBVFJBUFApLCBhIHJlc2VhcmNoIHByb2plY3QgZm9jdXNlZCBvbiB0aGUgcHJlZGljdGlvbiBhbmQgbWFuYWdlbWVudCBvZiBoYXJtZnVsIGN5YW5vYmFjdGVyaWFsIGJsb29tcy4gVGhlIHByZXNlbmNlIG9mIGN5YW5vdG94aW5zLCB3aGljaCBjYW4gY29uZmlybSB3aGV0aGVyIGEgbGFrZSBoYXMgYSB0b3hpYyBibG9vbSwgd2FzIGRldGVybWluZWQgdGhyb3VnaCBoaWdoLXJlc29sdXRpb24gbWFzcyBzcGVjdHJvbWV0cnkgYW5hbHlzZXMuIFRoaXMgcGFwZXIgcHJlc2VudHMgYW4gb3ZlcnZpZXcgb2YgdGhlIGltcGxlbWVudGF0aW9uIG9mIHRoZSBBZG9wdCBhIExha2UgaW5pdGlhdGl2ZSwgdGhlIGNhbXBhaWdu4oCZcyBzdGF0dXMsIGFuZCB0aGUgbGVzc29ucyBsZWFybmVkLCBhbmQgaXQgYXJndWVzIHRoZSBpbXBvcnRhbmNlIG9mIGNvbnRpbnVhbCBtb25pdG9yaW5nIG9mIGN5YW5vYmFjdGVyaWFsIGJsb29tcy4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuNTMzNC9jc3RwLjY1NSIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9zdG9yYWdlLmdvb2dsZWFwaXMuY29tL2pubC11cC1qLWNzdHAtZmlsZXMvam91cm5hbHMvMS9hcnRpY2xlcy82NTUvNjU4ZTlkN2Q0NmEwYy5wZGYiLCJpc3N1ZWREYXRlIjoiMjAyMy0xMi0yOSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDgiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkNpdGl6ZW4gU2NpZW5jZSBUaGVvcnkgYW5kIFByYWN0aWNlIiwiaXNzdWUiOjEsInZvbHVtZSI6OCwiaW1wYWN0RmFjdG9yIjoiMy4xMiJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjY2LCJsYXN0UGFnZSI6NjZ9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiVWJpcXVpdHkgUHJlc3MifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjI3LCJjaXRlZEJ5Q291bnQiOjJ9fSwicXVvdGUiOiJJbiB0aGUgVW5pdGVkICBTdGF0ZXMsIGEgdm9sdW50ZWVyIG1vbml0b3JpbmcgcHJvZ3JhbSBkZXZlbG9wZWQgYnkgdGhlICBXYWJhc2ggUml2ZXIgRW5oYW5jZW1lbnQgQ29ycG9yYXRpb24gaGVscGVkIHRvIGlkZW50aWZ5ICB3YXRlci1xdWFsaXR5IHByaW9yaXRpZXMgYWNyb3NzIHRoZSB3YXRlcnNoZWQgKE11ZW5pY2ggIGV0IGFsLiAyMDE2KS4gSW4gQ2FuYWRhLCBvcmdhbml6YXRpb25zIGFyZSBjb21taXR0ZWQgIHRvIG1vbml0b3IgYW5kIHByb3RlY3Qgd2F0ZXIgdGhyb3VnaCBjcm93ZHNvdXJjZWQgIG1vbml0b3JpbmcgYW5kIGtub3dsZWRnZSB0cmFuc2ZlciBpbml0aWF0aXZlcyAoRzNFICAyMDIyOyBPdHRhd2EgUml2ZXJrZWVwZXIgMjAyMzsgT3VyIExpdmluZyBXYXRlcnMgMjAyMjsgIFdhdGVyIFJhbmdlcnMgMjAyMjsgWmhhbywgTGl1IGFuZCBXZWkgMjAyMCkuVGhlc2UgIGVmZm9ydHMgYXJlIGNvbXBsZW1lbnRlZCBieSBvcGVuIGZyZWUgcGxhdGZvcm1zIHN1Y2ggIGFzIERhdGFTdHJlYW0sIHdoZXJlIHdhdGVyIHF1YWxpdHkgZGF0YSBjb2xsZWN0ZWQgZnJvbSAgdmFyaW91cyBjb21tdW5pdHktZ2F0aGVyZWQgcHJvZ3JhbXMgaXMgY3VyYXRlZCwgc3RvcmVkLCAgYW5kIHNoYXJlZCB3aXRoIHRoZSBwdWJsaWMgKFRoZSBHb3Jkb24gRm91bmRhdGlvbiAyMDIzKS4gR2l2ZW4gdGhlIGNyaXRpY2FsIGltcG9ydGFuY2Ugb2Ygd2F0ZXIgcmVzb3VyY2VzIGFuZCB0aGVpciBkaXZlcnNlIHVzZXMsIGluY2x1ZGluZyBkb21lc3RpYyB3ZWxscywgbXVuaWNpcGFsIGRyaW5raW5nIHdhdGVyIHByb2R1Y3Rpb24sIGZpc2hpbmcsIHJlY3JlYXRpb25hbCBhY3Rpdml0aWVzLCAgYW5kIGNyb3AgaXJyaWdhdGlvbiwgaXQgaXMgZXNzZW50aWFsIHRvIHByb3RlY3QgYW5kIG1vbml0b3IgIHRoZW0uIExha2VzLCBvZnRlbiB1c2VkIGZvciByZWNyZWF0aW9uYWwgYWN0aXZpdGllcyBhbmQgYXMgIGRyaW5raW5nIHdhdGVyIHNvdXJjZXMsIGFyZSBzdXNjZXB0aWJsZSB0byBibG9vbXMgb2YgYWxnYWUgIGFuZCBjeWFub2JhY3RlcmlhLCBwYXJ0aWN1bGFybHkgaW4gd2FybSwgc2xvdy1tb3ZpbmcsIGFuZCAgbnV0cmllbnQtcmljaCB3YXRlcnMuIFRoZXNlIGJsb29tcyBjYW4gbWFuaWZlc3QgYXMgZm9hbSwgIHNjdW0sIG1hdHMsIG9yIHBhaW50LWxpa2UgY29hdGluZ3Mgb24gdGhlIHdhdGVy4oCZcyBzdXJmYWNlICBhbmQgY2FuIHByb2R1Y2UgY3lhbm90b3hpbnMgdGhhdCBwb3NlIGhlYWx0aCByaXNrcyB0byAgaHVtYW5zIGFuZCB3aWxkbGlmZSAoRmlndXJlIDEpLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjEyLjQ2NDA3MjIyNzQ3ODAyNywibGVmdCI6NTEuODU0ODg4OTE2MDE1NjI1LCJ3aWR0aCI6MzguNzg2NzU0NjA4MTU0MywiaGVpZ2h0IjoxLjM1NjgxODQzNzU3NjI5NH19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjE0LjEwNDk3ODU2MTQwMTM2NywibGVmdCI6NTEuODUzNDE2NDQyODcxMDk0LCJ3aWR0aCI6MzguNzg5NjkxOTI1MDQ4ODMsImhlaWdodCI6MS4zNTY4MTg0Mzc1NzYyOTR9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoxNS43NDU4ODM5NDE2NTAzOSwibGVmdCI6NTEuODUzNDE2NDQyODcxMDk0LCJ3aWR0aCI6MzguNzg2NzM5MzQ5MzY1MjM0LCJoZWlnaHQiOjEuMzU2ODE4NDM3NTc2Mjk0fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MTcuMzg2Nzg5MzIxODk5NDE0LCJsZWZ0Ijo1MS44NTM0MTY0NDI4NzEwOTQsIndpZHRoIjozOC43ODgyMTU2MzcyMDcwMywiaGVpZ2h0IjoxLjM1NjgxODQzNzU3NjI5NH19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjE5LjAyNzY5NDcwMjE0ODQzOCwibGVmdCI6NTEuODUzNDE2NDQyODcxMDk0LCJ3aWR0aCI6MzguNzg4MjE1NjM3MjA3MDMsImhlaWdodCI6MS4zNTY4MTg0Mzc1NzYyOTR9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyMC42Njg2MDAwODIzOTc0NiwibGVmdCI6NTEuODUzNDE2NDQyODcxMDk0LCJ3aWR0aCI6MzguNzg1Mjc0NTA1NjE1MjM0LCJoZWlnaHQiOjEuMzU2ODE4NDM3NTc2Mjk0fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MjIuMzA5NTA1NDYyNjQ2NDg0LCJsZWZ0Ijo1MS44NTM0MTY0NDI4NzEwOTQsIndpZHRoIjozOC43ODgyMzQ3MTA2OTMzNiwiaGVpZ2h0IjoxLjM1NjgxODQzNzU3NjI5NH19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjIzLjk1MDQxMDg0Mjg5NTUwOCwibGVmdCI6NTEuODUzNDEyNjI4MTczODMsIndpZHRoIjozOC43ODM4MzI1NTAwNDg4MywiaGVpZ2h0IjoxLjM1NjgxODQzNzU3NjI5NH19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI1LjU5MTMxNjIyMzE0NDUzLCJsZWZ0Ijo1MS44NDkwMDY2NTI4MzIwMywid2lkdGgiOjM4Ljc5MTE3MjAyNzU4Nzg5LCJoZWlnaHQiOjEuMzU2ODE4NDM3NTc2Mjk0fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MjcuMjMyMjIxNjAzMzkzNTU1LCJsZWZ0Ijo1MS44NTE5NDAxNTUwMjkzLCJ3aWR0aCI6MzguNzkyNjE3Nzk3ODUxNTYsImhlaWdodCI6MS4zNTY4MTg0Mzc1NzYyOTR9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyOC44NzMxMjY5ODM2NDI1NzgsImxlZnQiOjUxLjg1MTk0MDE1NTAyOTMsIndpZHRoIjozOC43ODM3NzUzMjk1ODk4NDQsImhlaWdodCI6MS4zNTY4MTg0Mzc1NzYyOTR9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozMC41MTQwMzQyNzEyNDAyMzQsImxlZnQiOjUxLjg1MTk0MDE1NTAyOTMsIndpZHRoIjozOC43ODgyMzA4OTU5OTYwOTQsImhlaWdodCI6MS4zNTY4MTg0Mzc1NzYyOTR9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozMi4xNTQ5Mzc3NDQxNDA2MjUsImxlZnQiOjUxLjg1MTk0MDE1NTAyOTMsIndpZHRoIjozOC41ODUyODkwMDE0NjQ4NDQsImhlaWdodCI6MS4zNTY4MTg0Mzc1NzYyOTR9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozMy43OTU4NDUwMzE3MzgyOCwibGVmdCI6NTMuNzA0ODc5NzYwNzQyMTksIndpZHRoIjozNi45MzUyNzk4NDYxOTE0MDYsImhlaWdodCI6MS4zNTY4MTg0Mzc1NzYyOTR9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNS40MzY3NTIzMTkzMzU5NCwibGVmdCI6NTEuODUxOTQwMTU1MDI5Mywid2lkdGgiOjM4Ljc4OTcwNzE4MzgzNzg5LCJoZWlnaHQiOjEuMzU2ODE4NDM3NTc2Mjk0fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MzcuMDc3NjYzNDIxNjMwODYsImxlZnQiOjUxLjg1MTk0MDE1NTAyOTMsIndpZHRoIjozOC43ODY3NjYwNTIyNDYwOTQsImhlaWdodCI6MS4zNTY4MTg0Mzc1NzYyOTR9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC43MTg1NzA3MDkyMjg1MTYsImxlZnQiOjUxLjg1MTk0MDE1NTAyOTMsIndpZHRoIjozOC43ODM4MTM0NzY1NjI1LCJoZWlnaHQiOjEuMzU2ODE4NDM3NTc2Mjk0fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDAuMzU5NDgxODExNTIzNDQsImxlZnQiOjUxLjg1MTk0MDE1NTAyOTMsIndpZHRoIjozOC43ODY3NzM2ODE2NDA2MjUsImhlaWdodCI6MS4zNTY4MTg0Mzc1NzYyOTR9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Mi4wMDAzOTI5MTM4MTgzNiwibGVmdCI6NTEuODUxOTQwMTU1MDI5Mywid2lkdGgiOjM4Ljc4NTMwMTIwODQ5NjA5NCwiaGVpZ2h0IjoxLjM1NjgxODQzNzU3NjI5NH19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQzLjY0MTMwMDIwMTQxNjAxNiwibGVmdCI6NTEuODUxOTQwMTU1MDI5Mywid2lkdGgiOjM4Ljc5MTE1Mjk1NDEwMTU2LCJoZWlnaHQiOjEuMzU2ODE4NDM3NTc2Mjk0fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDUuMjgyMjA3NDg5MDEzNjcsImxlZnQiOjUxLjg1MTk0MDE1NTAyOTMsIndpZHRoIjozOC43ODk3MDcxODM4Mzc4OSwiaGVpZ2h0IjoxLjM1NjgxODQzNzU3NjI5NH19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQ2LjkyMzExODU5MTMwODU5NCwibGVmdCI6NTEuODUxOTQwMTU1MDI5Mywid2lkdGgiOjM4Ljc5MjYyOTI0MTk0MzM2LCJoZWlnaHQiOjEuMzU2ODE4NDM3NTc2Mjk0fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDguNTY0MDI5NjkzNjAzNTE2LCJsZWZ0Ijo1MS44NTE5NDAxNTUwMjkzLCJ3aWR0aCI6MzguNzg1Mjc4MzIwMzEyNSwiaGVpZ2h0IjoxLjM1NjgxODQzNzU3NjI5NH19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjUwLjIwNDkzNjk4MTIwMTE3LCJsZWZ0Ijo1MS44NTE5NDAxNTUwMjkzLCJ3aWR0aCI6MzguNzg2NzY2MDUyMjQ2MDk0LCJoZWlnaHQiOjEuMzU2ODE4NDM3NTc2Mjk0fX1dfSwiY2h1bmtJZCI6ImRmNDIwOGI2LTg0YjYtNDNkZS05MTdjLTJkOGVlZjhiZmMwNSIsImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjY3In19XQ%3D%3D.

Ultimately, citizen science democratizes HAB management, generating dense datasets that expose regulatory shortfalls like uncurbed non-point sources fueling internal loading and advocate for tailored BMPs (Khorasani & Zhu, 2021)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhhbWVkIiwiZmFtaWx5IjoiS2hvcmFzYW5pIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlpoZW5kdW8iLCJmYW1pbHkiOiJaaHUifX1dLCJpZHMiOnsiaXNzbiI6WyIwMDIyLTE2OTQiLCIxODc5LTI3MDciXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMTYvai5qaHlkcm9sLjIwMjEuMTI2ODg2IiwibWFnIjoiMzE3NzA3MTI5OCIsIm9wZW5hbGV4IjoiVzMxNzcwNzEyOTgifSwidGl0bGUiOiJQaG9zcGhvcnVzIHJldGVudGlvbiBpbiBsYWtlczogQSBjcml0aWNhbCByZWFzc2Vzc21lbnQgb2YgaHlwb3RoZXNlcyBhbmQgc3RhdGljIG1vZGVscyIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2ouamh5ZHJvbC4yMDIxLjEyNjg4NiIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9lYXJ0aGFyeGl2Lm9yZy9yZXBvc2l0b3J5L29iamVjdC8yNDgxL2Rvd25sb2FkLzUwNzQvIiwiaXNzdWVkRGF0ZSI6IjIwMjEtMDktMDIiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA4IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJKb3VybmFsIG9mIEh5ZHJvbG9neSIsInZvbHVtZSI6NjAzLCJpbXBhY3RGYWN0b3IiOiI3LjY0In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTI2ODg2LCJsYXN0UGFnZSI6MTI2ODg2fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkVsc2V2aWVyIEJWIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo3NywiY2l0ZWRCeUNvdW50IjoxMn19LCJxdW90ZSI6Ikhvd2V2ZXIsIGh1bWFuIGFjdGl2aXRpZXMgaGF2ZSA0OSBhY2NlbGVyYXRlZCB0aGUgZXV0cm9waGljYXRpb24gcHJvY2VzcyBieSBpbmNyZWFzaW5nIHRoZSBudXRyaWVudHMgZGVsaXZlcnkgdG8gdGhlIGFxdWF0aWMgc3lzdGVtcyA1MCAoTWVrb25uZW4gYW5kIEhvZWtzdHJhLCAyMDE4KS4gVGh1cywgYW50aHJvcG9nZW5pYyBldXRyb3BoaWNhdGlvbiBpcyBvbmUgb2YgdGhlIG1vc3QgaW1wb3J0YW50IDUxIGVsZW1lbnRzIG9mIGZyZXNoIGFuZCBtYXJpbmUgd2F0ZXIgcXVhbGl0eSBkZXRlcmlvcmF0aW9uIChIdSBldCBhbC4sIDIwMTk7IFNtaXRoIGFuZCBTY2hpbmRsZXIsIDUyIDIwMDkpLiBPbmUgZGlyZWN0IGNvbnNlcXVlbmNlIG9mIGFudGhyb3BvZ2VuaWMgZXV0cm9waGljYXRpb24gY29tZXMgaW4gdGhlIGZvcm0gb2YgbWFzc2l2ZSBhbGdhbCA1MyBibG9vbXMgKEdyYW7DqWxpIGV0IGFsLiwgMjAwODsgSGVpc2xlciBldCBhbC4sIDIwMDgpLCB3aGljaCBhcmUgcHJlZGljdGVkIHRvIGJlIGludGVuc2lmaWVkIHVuZGVyIDU0IHdhcm1lciB3YXRlciB0ZW1wZXJhdHVyZXMgYXMgY2xpbWF0ZSBjaGFuZ2VzIChHb2JsZXIsIDIwMjA7IE11a3VuZGFuIGV0IGFsLiwgMjAyMCkuIDU1IEV1dHJvcGhpY2F0aW9uIGlzIGEg4oCcd2lja2Vk4oCdIHByb2JsZW0sIHdoaWNoIGlzIHRoZSBjb25zZXF1ZW5jZSBvZiB2YXJpb3VzIHByb2Nlc3NlcyB0aGF0IG9wZXJhdGUgNTYgY3VtdWxhdGl2ZWx5LiBDb25zaWRlcmluZyB0aGUgdW5pcXVlbmVzcyBvZiBlYWNoIGxha2UgYW5kIGl0cyBzdXJyb3VuZGluZyBhcmVhLCB0aGVyZSBpcyBubyBicm9hZGx5IDU3IGFwcGxpY2FibGUgc2V0IG9mIGJlc3QgbWFuYWdlbWVudCBwcmFjdGljZXMgdGhhdCBjYW4gYmUgYXBwbGllZCBpbiB3YXRlcnNoZWRzIHRvIG1pdGlnYXRlIDU4IHBob3NwaG9ydXMgbG9hZGluZyBhbmQgaXRzIGltcGFjdCBvbiBhbGwgbGFrZXMgKFRob3JudG9uIGV0IGFsLiwgMjAxMykuIEhlbmNlLCBldXRyb3BoaWNhdGlvbiA1OSBtYW5hZ2VtZW50IGFuZCBsYWtlIHJlc3RvcmF0aW9uIG5lZWQgaW50ZWdyYXRlZCBwbGFucyB0aGF0IGFyZSBub3Qgb25seSBzY2llbnRpZmljYWxseSB2YWxpZCBidXQgNjAgYWxzbyBzb2Npby1lY29ub21pY2FsbHkgc2F0aXNmeWluZyAoR2lic29uIGV0IGFsLiwgMjAwMCkuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MzYuMTY4NDM0MTQzMDY2NDA2LCJsZWZ0IjoxMS43Njg2MjgxMjA0MjIzNjMsIndpZHRoIjo3Ni45NjMzOTQxNjUwMzkwNiwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjM2LjcxODQzMzM4MDEyNjk1LCJsZWZ0Ijo2Ljk4MDM5MjQ1NjA1NDY4NzUsIndpZHRoIjoyLjI1MDk1NzQ4OTAxMzY3MiwiaGVpZ2h0IjoxLjM5Mzk0MDQ0ODc2MDk4NjN9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjozOS42NTMyODU5ODAyMjQ2MSwibGVmdCI6Ni45ODAzOTI0NTYwNTQ2ODc1LCJ3aWR0aCI6ODEuNzYxNDI4ODMzMDA3ODEsImhlaWdodCI6Mi4wMjEyMTE2MjQxNDU1MDh9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo0My4xMzgxMzQwMDI2ODU1NSwibGVmdCI6Ni45ODAzOTI0NTYwNTQ2ODc1LCJ3aWR0aCI6ODEuNzU1NTYxODI4NjEzMjgsImhlaWdodCI6Mi4wMjEyMTE2MjQxNDU1MDh9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo0Ni42MjI5ODIwMjUxNDY0ODQsImxlZnQiOjYuOTgwMzkyNDU2MDU0Njg3NSwid2lkdGgiOjgxLjc0MjgwNTQ4MDk1NzAzLCJoZWlnaHQiOjIuMDIxMjExNjI0MTQ1NTA4fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6NTAuMTA3ODMwMDQ3NjA3NDIsImxlZnQiOjYuOTgwMzkyNDU2MDU0Njg3NSwid2lkdGgiOjgxLjc1MjYxNjg4MjMyNDIyLCJoZWlnaHQiOjIuMDIxMjExNjI0MTQ1NTA4fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6NTMuNTk1MjAzMzk5NjU4MiwibGVmdCI6Ni45ODAzOTI0NTYwNTQ2ODc1LCJ3aWR0aCI6ODEuNzQ4NzAzMDAyOTI5NjksImhlaWdodCI6Mi4wMjEyMTE2MjQxNDU1MDh9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo1Ny4wODAwNDc2MDc0MjE4NzUsImxlZnQiOjYuOTgwMzkyNDU2MDU0Njg3NSwid2lkdGgiOjczLjMxMDQ1NTMyMjI2NTYyLCJoZWlnaHQiOjIuMDIxMjExNjI0MTQ1NTA4fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6NjEuNTY0OTAzMjU5Mjc3MzQ0LCJsZWZ0Ijo2Ljk4MDM5MjQ1NjA1NDY4NzUsIndpZHRoIjo4MS43NTc1MTQ5NTM2MTMyOCwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjY1LjA0OTc0MzY1MjM0Mzc1LCJsZWZ0Ijo2Ljk4MDM5MjQ1NjA1NDY4NzUsIndpZHRoIjo4MS43NTk0NzU3MDgwMDc4MSwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjY4LjUzNDU5MTY3NDgwNDY5LCJsZWZ0Ijo2Ljk4MDM5MjQ1NjA1NDY4NzUsIndpZHRoIjo4MS43MDM5MzM3MTU4MjAzMSwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjcyLjAxOTQ1NDk1NjA1NDY5LCJsZWZ0Ijo2Ljk4MDM5MjQ1NjA1NDY4NzUsIndpZHRoIjo4MS43NTA2NTYxMjc5Mjk2OSwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjc1LjUwODA4NzE1ODIwMzEyLCJsZWZ0Ijo2Ljk4MDM5MjQ1NjA1NDY4NzUsIndpZHRoIjo4MS43NTc1MTQ5NTM2MTMyOCwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjc4Ljk5MjkzNTE4MDY2NDA2LCJsZWZ0IjoxMS43Njg2MjgxMjA0MjIzNjMsIndpZHRoIjo3Ni45NzMxOTc5MzcwMTE3MiwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEyNjg4OCJ9fV0%3D. By fostering early detection, it prevents cascading tolls: biodiversity loss, toxin risks, and socio-economic hits from tourism slumps (Devlin et al., 2020; Sampat et al., 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFwb29ydmEgTS4iLCJmYW1pbHkiOiJTYW1wYXQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW5kcmVhIiwiZmFtaWx5IjoiSGlja3MifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiR2VyYXJkbyBKLiIsImZhbWlseSI6IlJ1aXrigJBNZXJjYWRvIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlbDrcKtY3RvciBNLiIsImZhbWlseSI6IlphdmFsYSJ9fV0sImlkcyI6eyJpc3NuIjpbIjA5MjEtMzQ0OSIsIjE4NzktMDY1OCJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMTAxNi9qLnJlc2NvbnJlYy4yMDIwLjEwNTE5OSIsIm1hZyI6IjMwOTQ2MjUxMDQiLCJwbWlkIjoiMzM3NDYzNjEiLCJvcGVuYWxleCI6IlczMDk0NjI1MTA0In0sInRpdGxlIjoiVmFsdWluZyBlY29ub21pYyBpbXBhY3QgcmVkdWN0aW9ucyBvZiBudXRyaWVudCBwb2xsdXRpb24gZnJvbSBsaXZlc3RvY2sgd2FzdGUiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAxNi9qLnJlc2NvbnJlYy4yMDIwLjEwNTE5OSIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cubmNiaS5ubG0ubmloLmdvdi9wbWMvYXJ0aWNsZXMvNzk3MDUwNSIsImlzc3VlZERhdGUiOiIyMDIwLTEwLTE1IiwiYWNjZXNzZWREYXRlIjoiMjAyNi0wMSIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiUmVzb3VyY2VzIENvbnNlcnZhdGlvbiBhbmQgUmVjeWNsaW5nIiwidm9sdW1lIjoxNjQsImltcGFjdEZhY3RvciI6IjExLjAxIn0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTA1MTk5LCJsYXN0UGFnZSI6MTA1MTk5fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkVsc2V2aWVyIEJWIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjozOSwiY2l0ZWRCeUNvdW50Ijo1N319LCJxdW90ZSI6IlNlY3Rpb24gVGl0bGU6IFZhbHVpbmcgZWNvbm9taWMgaW1wYWN0IHJlZHVjdGlvbnMgb2YgbnV0cmllbnQgcG9sbHV0aW9uIGZyb20gbGl2ZXN0b2NrIHdhc3RlID4gMS4gSW50cm9kdWN0aW9uXG5Db250ZW50OlxuSW4gdGhlIFN0YXRlIG9mIEZsb3JpZGEsIEhBQnMgaGF2ZSByZXN1bHRlZCBpbiBhIG1vbnRobHkgbG9zcyBvZiAyLjggYW5kIDMuNyBtaWxsaW9uIFVTRCBjb3JyZXNwb25kaW5nIHRvIHJlc3RhdXJhbnQgYW5kIGxvZGdpbmcgcmV2ZW51ZSwgcmVzcGVjdGl2ZWx5ICggW0xhcmtpbiBhbmQgQWRhbXMsIDIwMDddKCkgKS4gRnJlcXVlbnQgb2NjdXJyZW5jZSBvZiBIQUJzIGFsc28gbG93ZXJzIHByb3BlcnR5IHZhbHVlcyBvZiBsYWtlZnJvbnQgcHJvcGVydGllcy4gVGhlIGxvc3MgaW4gcHJvcGVydHkgdmFsdWVzIGFyZSB0aGUgbGFyZ2VzdCBpbXBhY3QgYmVhcmVycyBvZiBldXRyb3BoaWNhdGlvbiB3aXRoIGFuIGVzdGltYXRlZCBhdmVyYWdlIGVjb25vbWljIGxvc3Mgb2YgMS42IGJpbGxpb24gVVNEIGFubnVhbGx5ICggW0RvZGRzIGV0IGFsLiwgMjAwOV0oKSApLiJ9LHsib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjEsInN1YnR5cGUiOnt9LCJjb250cmlidXRvcnMiOlt7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBZGFtIFQuIiwiZmFtaWx5IjoiRGV2bGluIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkppYXlpIiwiZmFtaWx5IjoiUGFuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik0uIE1hbmp1ciIsImZhbWlseSI6IlNoYWgifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQWxpeXUiLCJmYW1pbHkiOiJOdWh1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Iktlbm5ldGggVy4iLCJmYW1pbHkiOiJDdW1taW5zIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkNsYXVkaW8gVmFsZG92aW5vcyIsImZhbWlseSI6IlphcmdlcyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQYWJsbyIsImZhbWlseSI6IkZpZXJybyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJWaXZpYW5hIiwiZmFtaWx5IjoiT2xtb3MifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQWJkdWwtUmFobWFuIiwiZmFtaWx5IjoiRGlyaXN1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkpvaG4gT3ZpZSIsImZhbWlseSI6Ik9sb211a29ybyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJOYXRpdmlkYWRlIiwiZmFtaWx5IjoiVmllaXJhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFuYSIsImZhbWlseSI6IlNvYXJlcyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJDYXJsb3MiLCJmYW1pbHkiOiJQaW5oZWlybyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNYXR0aGV3IEYuIiwiZmFtaWx5IjoiQmluZ2hhbSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJKYXNvbiIsImZhbWlseSI6Iktpbm5lbGwifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiV3UiLCJmYW1pbHkiOiJCbyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJKaW5tdSIsImZhbWlseSI6IlpoYW5nIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllpbmRpIiwiZmFtaWx5IjoiWmhhbyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJIdW1iZXJ0byBBbHZlcyIsImZhbWlseSI6IkJhcmJvc2EifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiRnJhbmtsaW4iLCJmYW1pbHkiOiJQYXJlZGVz4oCQVHJlam8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiVCBMYWtzaG1pIiwiZmFtaWx5IjoiS3VtYXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiS2lyYW4iLCJmYW1pbHkiOiJUaGFrdXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQ2F0YXJpbmEiLCJmYW1pbHkiOiJCdXJpdGkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2NvdHQgQS4iLCJmYW1pbHkiOiJXZWxscyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJPc2NhciBPbW9uZGkiLCJmYW1pbHkiOiJEb25kZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJDYXJlbiBOYXZhbGlhIiwiZmFtaWx5IjoiQXRhbGl0c2EifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiRC4iLCJmYW1pbHkiOiJTYW5pIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNhbXBzb24iLCJmYW1pbHkiOiJBaG1lZCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBbWFuaW5nIiwiZmFtaWx5IjoiS3dha3UifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiR2V2b3JnIiwiZmFtaWx5IjoiQWRqZWkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2F0YXJ1cGEiLCJmYW1pbHkiOiJTaW1vbnlhbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTbmlnZGhhIiwiZmFtaWx5IjoiR2hvc2gifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUHJhc2FubmEiLCJmYW1pbHkiOiJDaGF0dGVyamVlIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNoaXZhIiwiZmFtaWx5IjoiR2hvcmEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQmVsZGEiLCJmYW1pbHkiOiJQcmFzYWQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiS2V0aGxhdGxvZ2lsZSIsImZhbWlseSI6Ik1vc2VwZWxlIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ill1c3VmIiwiZmFtaWx5IjoiU3VsZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJVaXLDoSIsImZhbWlseSI6Ik9saXZlaXJhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1hcmlhIEpvw6NvIiwiZmFtaWx5IjoiVmllaXJhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkZsZXVyIiwiZmFtaWx5IjoiRGUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTWluIiwiZmFtaWx5IjoiTWF4In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikdldm9yZyIsImZhbWlseSI6IlNpbW9ueWFuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNhdGFydXBhIiwiZmFtaWx5IjoiR2hvc2gifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU25pZ2RoYSIsImZhbWlseSI6IkNoYXR0ZXJqZWUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUHJhc2FubmEiLCJmYW1pbHkiOiJQcmFzYWQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2F0YXJ1cGEiLCJmYW1pbHkiOiJHaG9zaCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJHaG9yYSIsImZhbWlseSI6IlByYXNhZCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQcmFzYW5uYSIsImZhbWlseSI6IlBhbCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLd2FrdSIsImZhbWlseSI6IkFnb2R6byJ9fV0sImlkcyI6eyJpc3NuIjpbXSwiaXNibiI6W10sImRvaSI6IjEwLjU3NzIvaW50ZWNob3Blbi44NzQ2MyIsIm1hZyI6IjMxMjg1ODc0NjkiLCJvcGVuYWxleCI6IlczMTI4NTg3NDY5In0sInRpdGxlIjoiSW5sYW5kIFdhdGVycyAtIER5bmFtaWNzIGFuZCBFY29sb2d5IiwiYWJzdHJhY3QiOiJJbmxhbmQgd2F0ZXJzLCBsYWtlcywgcml2ZXJzLCBhbmQgdGhlaXIgY29ubmVjdGVkIHdldGxhbmRzIGFyZSB0aGUgbW9zdCBpbXBvcnRhbnQgYW5kIHRoZSBtb3N0IHZ1bG5lcmFibGUgc291cmNlcyBvZiBmcmVzaHdhdGVyIG9uIHRoZSBwbGFuZXQuIFRoZSBlY29sb2d5IG9mIHRoZXNlIHN5c3RlbXMgaW5jbHVkZXMgYmlvbG9neSBhcyB3ZWxsIGFzIGh1bWFuIHBvcHVsYXRpb25zIGFuZCBjaXZpbGl6YXRpb24uIElubGFuZCB3YXRlcnMgYW5kIHdldGxhbmRzIGFyZSBoaWdobHkgc3VzY2VwdGlibGUgdG8gY2hlbWljYWwgYW5kIGJpb2xvZ2ljYWwgcG9sbHV0YW50cyBmcm9tIG5hdHVyYWwgb3IgaHVtYW4gc291cmNlcywgY2hhbmdlcyBpbiB3YXRlcnNoZWQgZHluYW1pY3MgZHVlIHRvIHRoZSBlc3RhYmxpc2htZW50IG9mIGRhbXMgYW5kIHJlc2Vydm9pcnMsIGFuZCBsYW5kIHVzZSBjaGFuZ2VzIGZyb20gYWdyaWN1bHR1cmUgYW5kIGluZHVzdHJ5LiBUaGlzIGJvb2sgcHJvdmlkZXMgYSBjb21wcmVoZW5zaXZlIHJldmlldyBvZiBpc3N1ZXMgaW52b2x2aW5nIGlubGFuZCB3YXRlcnMgYW5kIGRpc2N1c3NlcyBtYW55IHdvcmxkd2lkZSBpbmxhbmQgd2F0ZXIgc3lzdGVtcy4gVGhlIG1haW4gdG9waWNzIG9mIHRoaXMgdGV4dCBhcmUgd2F0ZXIgcXVhbGl0eSBpbnZlc3RpZ2F0aW9uLCBhbmFseXNlcyBvZiB0aGUgZWNvbG9neSBvZiBpbmxhbmQgd2F0ZXIgc3lzdGVtcywgcmVtb3RlIHNlbnNpbmcgb2JzZXJ2YXRpb24gYW5kIG51bWVyaWNhbCBtb2RlbGluZyBtZXRob2RzLCBhbmQgYmlvZGl2ZXJzaXR5IGludmVzdGlnYXRpb25zLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC41NzcyL2ludGVjaG9wZW4uODc0NjMiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vbXRzLmludGVjaG9wZW4uY29tL3N0b3JhZ2UvYm9va3MvOTY2MC9hdXRob3JzX2Jvb2svYXV0aG9yc19ib29rLnBkZiIsImlzc3VlZERhdGUiOiIyMDIwLTA0LTAxIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiSW50ZWNoT3BlbiBlQm9va3MiLCJpbXBhY3RGYWN0b3IiOiIxLjAxIn0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkludGVjaE9wZW4ifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjMzOCwiY2l0ZWRCeUNvdW50IjoyN319LCJxdW90ZSI6IkFkZGl0aW9uYWxseSwgc2luY2UgYWZmZWN0ZWQgYnVzaW5lc3Nlc%2BKAmSB2YWx1ZXMgIGFyZSBtb3N0IGxpa2VseSB0aWVkIHRvIHRoZWlyIGFzc2V0cyBhbmQgdGhlIHJlYWwgZXN0YXRlIHRoZXkgb2NjdXB5IChmb3IgZXhhbXBsZSwgIGEgbWFyaW5hIGlzIG5vdCBlYXNpbHkgY29udmVydGVkIHRvIHNvbWUgb3RoZXIgdXNlKSwgb24tZ29pbmcgYmFsYW5jZSBzaGVldCBlZmZlY3RzICB3b3VsZCB1bHRpbWF0ZWx5IGxlYWQgdG8gcmVkdWN0aW9ucyBpbiBjb21tZXJjaWFsIHJlYWwgZXN0YXRlIHZhbHVlcy4gVGhlcmUgYXJlIG1hbnkgY2hhbGxlbmdlcyB0byB1bmRlcnN0YW5kaW5nIHRoZSBpbXBsaWNhdGlvbnMgb2YgY2hhbmdlcyBpbiAgdG91cmlzbSBmcm9tIEhBQnMuIFRoZSBjbGVhcmVzdCBjaGFsbGVuZ2Ugb2JzdHJ1Y3RpbmcgYSBwcmVjaXNlIGFuYWx5c2lzIG9mIHRoZXNlICBpbXBhY3RzIGlzIGEgbGFjayBvZiBkYXRhIGVpdGhlciBvbiB0aGUgYW1vdW50IG9mIHRvdXJpc20gYXQgcmlzayBvciB0aGUgc3BlY2lmaWMgIGltcGFjdCBvZiBIQUJzIG9uIHRvdXJpc20uIEZvciBleGFtcGxlLCB3aGlsZSBjb3VudHktbGV2ZWwgZGF0YSBleGlzdHMgZm9yIHRvdGFsICBleHBlbmRpdHVyZXMgb24gdG91cmlzbSwgdGhpcyBpbmNsdWRlcyB0b3VyaXNtIHdoaWNoIHdvdWxkIG5vdCBiZSBpbnRlcnJ1cHRlZCBieSAgSEFCcyBvciBvdGhlciBkaXNjb3VyYWdpbmcgZmFjdG9ycy4gQW4gYWRkaXRpb25hbCBjaGFsbGVuZ2UgcmVsYXRlcyB0byB0aGUgZGlzdGluY3Rpb24gYmV0d2VlbiBlY29ub21pYyBiZW5lZml0cyAgKHdpbGxpbmduZXNzIHRvIHBheSkgYW5kIGVjb25vbWljIGltcGFjdHMgKGV4cGVuZGl0dXJlcyksIGFuZCB0aGUgbWVhc3VyZS1tZW50IG9mIHRoZSBlY29ub21pYyBiZW5lZml0cyB0aGF0IGFyaXNlIGZyb20gZWNvbm9taWMgaW1wYWN0cyAocHJvZml0cykuIEZvciAgZXhhbXBsZSwgY29uc2lkZXIgYSByZXN0YXVyYW50IG93bmVyIHdobyBsb3NlcyAkMTAsMDAwIGluIHJldmVudWUgYmVjYXVzZSBvZiBhICBIQUIuIFRoZSBvd25lcuKAmXMgd2lsbGluZ25lc3MgdG8gcGF5IHRvIHJlY292ZXIgdGhhdCByZXZlbnVlOyBpcyAocm91Z2hseSBzcGVha2luZykgIHRoZSBsb3N0IHByb2ZpdCBvbiB0aGF0IHJldmVudWUuIFRoaXMgaXMgbW9yZSBkaWZmaWN1bHQgdG8gaWRlbnRpZnkgdGhhbiBsb3N0IHJldmVudWUuICAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6ODksImJib3giOnsidG9wIjo1OS40NDI4NjcyNzkwNTI3MzQsImxlZnQiOjE0LjcyMjg1MzY2MDU4MzQ5Niwid2lkdGgiOjY4Ljc0ODAwMTA5ODYzMjgxLCJoZWlnaHQiOjEuOTA1MDAwODA1ODU0Nzk3NH19LHsicGFnZU51bWJlciI6ODksImJib3giOnsidG9wIjo2MS4wNzEwNzU0Mzk0NTMxMjUsImxlZnQiOjE0LjcyMjg1MzY2MDU4MzQ5Niwid2lkdGgiOjY2LjM5NjM0NzA0NTg5ODQ0LCJoZWlnaHQiOjEuOTA1MDAwODA1ODU0Nzk3NH19LHsicGFnZU51bWJlciI6ODksImJib3giOnsidG9wIjo2Mi42OTkyNzU5NzA0NTg5ODQsImxlZnQiOjE0LjcyMjg1MzY2MDU4MzQ5Niwid2lkdGgiOjY5LjI2OTMwOTk5NzU1ODYsImhlaWdodCI6MS45MDUwMDA4MDU4NTQ3OTc0fX0seyJwYWdlTnVtYmVyIjo4OSwiYmJveCI6eyJ0b3AiOjY0LjMyNzQ4NDEzMDg1OTM4LCJsZWZ0IjoxNC43MjI4NTM2NjA1ODM0OTYsIndpZHRoIjo1Ny40MDA1MDUwNjU5MTc5NywiaGVpZ2h0IjoxLjkwNTAwMDgwNTg1NDc5NzR9fSx7InBhZ2VOdW1iZXIiOjg5LCJiYm94Ijp7InRvcCI6NjUuOTU1Njg4NDc2NTYyNSwibGVmdCI6MTcuNSwid2lkdGgiOjYzLjA5NTY5OTMxMDMwMjczNCwiaGVpZ2h0IjoxLjkwNTAwMDgwNTg1NDc5NzR9fSx7InBhZ2VOdW1iZXIiOjg5LCJiYm94Ijp7InRvcCI6NjcuNTgzODkyODIyMjY1NjIsImxlZnQiOjE0LjcyMjg1MzY2MDU4MzQ5Niwid2lkdGgiOjY4LjUyMjU5ODI2NjYwMTU2LCJoZWlnaHQiOjEuOTA0OTk2NjMzNTI5NjYzfX0seyJwYWdlTnVtYmVyIjo4OSwiYmJveCI6eyJ0b3AiOjY5LjIxMjA4OTUzODU3NDIyLCJsZWZ0IjoxNC43MjI4NTM2NjA1ODM0OTYsIndpZHRoIjo2NS40MTIyNzcyMjE2Nzk2OSwiaGVpZ2h0IjoxLjkwNTAwMDgwNTg1NDc5NzR9fSx7InBhZ2VOdW1iZXIiOjg5LCJiYm94Ijp7InRvcCI6NzAuODQwMjkzODg0Mjc3MzQsImxlZnQiOjE0LjcyMjg1MzY2MDU4MzQ5Niwid2lkdGgiOjY3Ljc3MDAxOTUzMTI1LCJoZWlnaHQiOjEuOTA1MDA1MDk3Mzg5MjIxMn19LHsicGFnZU51bWJlciI6ODksImJib3giOnsidG9wIjo3Mi40Njg0OTgyMjk5ODA0NywibGVmdCI6MTQuNzIyODUzNjYwNTgzNDk2LCJ3aWR0aCI6NjkuMzg2MzI5NjUwODc4OSwiaGVpZ2h0IjoxLjkwNTAwNTA5NzM4OTIyMTJ9fSx7InBhZ2VOdW1iZXIiOjg5LCJiYm94Ijp7InRvcCI6NzQuMDk2NzAyNTc1NjgzNiwibGVmdCI6MTQuNzIyODUzNjYwNTgzNDk2LCJ3aWR0aCI6MzAuMTM0ODgwMDY1OTE3OTcsImhlaWdodCI6MS45MDUwMDUwOTczODkyMjEyfX0seyJwYWdlTnVtYmVyIjo4OSwiYmJveCI6eyJ0b3AiOjc1LjcyNDkxNDU1MDc4MTI1LCJsZWZ0IjoxNy41LCJ3aWR0aCI6NjQuNDEwNzk3MTE5MTQwNjIsImhlaWdodCI6MS45MDUwMDUwOTczODkyMjEyfX0seyJwYWdlTnVtYmVyIjo4OSwiYmJveCI6eyJ0b3AiOjc3LjM1MzExMTI2NzA4OTg0LCJsZWZ0IjoxNC43MjI4NTM2NjA1ODM0OTYsIndpZHRoIjo2NC44NTU2OTAwMDI0NDE0LCJoZWlnaHQiOjEuOTA1MDA1MDk3Mzg5MjIxMn19LHsicGFnZU51bWJlciI6ODksImJib3giOnsidG9wIjo3OC45ODEzMTU2MTI3OTI5NywibGVmdCI6MTQuNzIyODUzNjYwNTgzNDk2LCJ3aWR0aCI6NjYuMDE5ODUxNjg0NTcwMzEsImhlaWdodCI6MS45MDUwMDUwOTczODkyMjEyfX0seyJwYWdlTnVtYmVyIjo4OSwiYmJveCI6eyJ0b3AiOjgwLjYwOTUyNzU4Nzg5MDYyLCJsZWZ0IjoxNC43MjI4NTM2NjA1ODM0OTYsIndpZHRoIjo2Ny43ODc2NDM0MzI2MTcxOSwiaGVpZ2h0IjoxLjkwNTAwNTA5NzM4OTIyMTJ9fSx7InBhZ2VOdW1iZXIiOjg5LCJiYm94Ijp7InRvcCI6ODIuMjM3NzI0MzA0MTk5MjIsImxlZnQiOjE0LjcyMjg1MzY2MDU4MzQ5Niwid2lkdGgiOjY5LjczOTY5MjY4Nzk4ODI4LCJoZWlnaHQiOjEuOTA1MDA1MDk3Mzg5MjIxMn19LHsicGFnZU51bWJlciI6ODksImJib3giOnsidG9wIjo4My44NjU5MzYyNzkyOTY4OCwibGVmdCI6MTQuNzIyODUzNjYwNTgzNDk2LCJ3aWR0aCI6NjguNDg3MTI5MjExNDI1NzgsImhlaWdodCI6MS45MDUwMDUwOTczODkyMjEyfX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiOTAifX1d. As India transitions to adaptive governance, scaling these initiatives via state wetland authorities promises sustained resilience, paving the way for indigenous knowledge revival and nature-based solutions in subsequent strategies.

13. Integrating Traditional Ecological Knowledge in Water Management

Complementing citizen science with traditional ecological knowledge offers a culturally resonant pathway to revive indigenous water management practices, addressing the "wicked" challenges of eutrophication and HABs in Indian lakes (Khorasani & Zhu, 2021)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhhbWVkIiwiZmFtaWx5IjoiS2hvcmFzYW5pIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlpoZW5kdW8iLCJmYW1pbHkiOiJaaHUifX1dLCJpZHMiOnsiaXNzbiI6WyIwMDIyLTE2OTQiLCIxODc5LTI3MDciXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMTYvai5qaHlkcm9sLjIwMjEuMTI2ODg2IiwibWFnIjoiMzE3NzA3MTI5OCIsIm9wZW5hbGV4IjoiVzMxNzcwNzEyOTgifSwidGl0bGUiOiJQaG9zcGhvcnVzIHJldGVudGlvbiBpbiBsYWtlczogQSBjcml0aWNhbCByZWFzc2Vzc21lbnQgb2YgaHlwb3RoZXNlcyBhbmQgc3RhdGljIG1vZGVscyIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2ouamh5ZHJvbC4yMDIxLjEyNjg4NiIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9lYXJ0aGFyeGl2Lm9yZy9yZXBvc2l0b3J5L29iamVjdC8yNDgxL2Rvd25sb2FkLzUwNzQvIiwiaXNzdWVkRGF0ZSI6IjIwMjEtMDktMDIiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA4IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJKb3VybmFsIG9mIEh5ZHJvbG9neSIsInZvbHVtZSI6NjAzLCJpbXBhY3RGYWN0b3IiOiI3LjY0In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTI2ODg2LCJsYXN0UGFnZSI6MTI2ODg2fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkVsc2V2aWVyIEJWIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo3NywiY2l0ZWRCeUNvdW50IjoxMn19LCJxdW90ZSI6Ikhvd2V2ZXIsIGh1bWFuIGFjdGl2aXRpZXMgaGF2ZSA0OSBhY2NlbGVyYXRlZCB0aGUgZXV0cm9waGljYXRpb24gcHJvY2VzcyBieSBpbmNyZWFzaW5nIHRoZSBudXRyaWVudHMgZGVsaXZlcnkgdG8gdGhlIGFxdWF0aWMgc3lzdGVtcyA1MCAoTWVrb25uZW4gYW5kIEhvZWtzdHJhLCAyMDE4KS4gVGh1cywgYW50aHJvcG9nZW5pYyBldXRyb3BoaWNhdGlvbiBpcyBvbmUgb2YgdGhlIG1vc3QgaW1wb3J0YW50IDUxIGVsZW1lbnRzIG9mIGZyZXNoIGFuZCBtYXJpbmUgd2F0ZXIgcXVhbGl0eSBkZXRlcmlvcmF0aW9uIChIdSBldCBhbC4sIDIwMTk7IFNtaXRoIGFuZCBTY2hpbmRsZXIsIDUyIDIwMDkpLiBPbmUgZGlyZWN0IGNvbnNlcXVlbmNlIG9mIGFudGhyb3BvZ2VuaWMgZXV0cm9waGljYXRpb24gY29tZXMgaW4gdGhlIGZvcm0gb2YgbWFzc2l2ZSBhbGdhbCA1MyBibG9vbXMgKEdyYW7DqWxpIGV0IGFsLiwgMjAwODsgSGVpc2xlciBldCBhbC4sIDIwMDgpLCB3aGljaCBhcmUgcHJlZGljdGVkIHRvIGJlIGludGVuc2lmaWVkIHVuZGVyIDU0IHdhcm1lciB3YXRlciB0ZW1wZXJhdHVyZXMgYXMgY2xpbWF0ZSBjaGFuZ2VzIChHb2JsZXIsIDIwMjA7IE11a3VuZGFuIGV0IGFsLiwgMjAyMCkuIDU1IEV1dHJvcGhpY2F0aW9uIGlzIGEg4oCcd2lja2Vk4oCdIHByb2JsZW0sIHdoaWNoIGlzIHRoZSBjb25zZXF1ZW5jZSBvZiB2YXJpb3VzIHByb2Nlc3NlcyB0aGF0IG9wZXJhdGUgNTYgY3VtdWxhdGl2ZWx5LiBDb25zaWRlcmluZyB0aGUgdW5pcXVlbmVzcyBvZiBlYWNoIGxha2UgYW5kIGl0cyBzdXJyb3VuZGluZyBhcmVhLCB0aGVyZSBpcyBubyBicm9hZGx5IDU3IGFwcGxpY2FibGUgc2V0IG9mIGJlc3QgbWFuYWdlbWVudCBwcmFjdGljZXMgdGhhdCBjYW4gYmUgYXBwbGllZCBpbiB3YXRlcnNoZWRzIHRvIG1pdGlnYXRlIDU4IHBob3NwaG9ydXMgbG9hZGluZyBhbmQgaXRzIGltcGFjdCBvbiBhbGwgbGFrZXMgKFRob3JudG9uIGV0IGFsLiwgMjAxMykuIEhlbmNlLCBldXRyb3BoaWNhdGlvbiA1OSBtYW5hZ2VtZW50IGFuZCBsYWtlIHJlc3RvcmF0aW9uIG5lZWQgaW50ZWdyYXRlZCBwbGFucyB0aGF0IGFyZSBub3Qgb25seSBzY2llbnRpZmljYWxseSB2YWxpZCBidXQgNjAgYWxzbyBzb2Npby1lY29ub21pY2FsbHkgc2F0aXNmeWluZyAoR2lic29uIGV0IGFsLiwgMjAwMCkuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MzYuMTY4NDM0MTQzMDY2NDA2LCJsZWZ0IjoxMS43Njg2MjgxMjA0MjIzNjMsIndpZHRoIjo3Ni45NjMzOTQxNjUwMzkwNiwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjM2LjcxODQzMzM4MDEyNjk1LCJsZWZ0Ijo2Ljk4MDM5MjQ1NjA1NDY4NzUsIndpZHRoIjoyLjI1MDk1NzQ4OTAxMzY3MiwiaGVpZ2h0IjoxLjM5Mzk0MDQ0ODc2MDk4NjN9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjozOS42NTMyODU5ODAyMjQ2MSwibGVmdCI6Ni45ODAzOTI0NTYwNTQ2ODc1LCJ3aWR0aCI6ODEuNzYxNDI4ODMzMDA3ODEsImhlaWdodCI6Mi4wMjEyMTE2MjQxNDU1MDh9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo0My4xMzgxMzQwMDI2ODU1NSwibGVmdCI6Ni45ODAzOTI0NTYwNTQ2ODc1LCJ3aWR0aCI6ODEuNzU1NTYxODI4NjEzMjgsImhlaWdodCI6Mi4wMjEyMTE2MjQxNDU1MDh9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo0Ni42MjI5ODIwMjUxNDY0ODQsImxlZnQiOjYuOTgwMzkyNDU2MDU0Njg3NSwid2lkdGgiOjgxLjc0MjgwNTQ4MDk1NzAzLCJoZWlnaHQiOjIuMDIxMjExNjI0MTQ1NTA4fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6NTAuMTA3ODMwMDQ3NjA3NDIsImxlZnQiOjYuOTgwMzkyNDU2MDU0Njg3NSwid2lkdGgiOjgxLjc1MjYxNjg4MjMyNDIyLCJoZWlnaHQiOjIuMDIxMjExNjI0MTQ1NTA4fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6NTMuNTk1MjAzMzk5NjU4MiwibGVmdCI6Ni45ODAzOTI0NTYwNTQ2ODc1LCJ3aWR0aCI6ODEuNzQ4NzAzMDAyOTI5NjksImhlaWdodCI6Mi4wMjEyMTE2MjQxNDU1MDh9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo1Ny4wODAwNDc2MDc0MjE4NzUsImxlZnQiOjYuOTgwMzkyNDU2MDU0Njg3NSwid2lkdGgiOjczLjMxMDQ1NTMyMjI2NTYyLCJoZWlnaHQiOjIuMDIxMjExNjI0MTQ1NTA4fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6NjEuNTY0OTAzMjU5Mjc3MzQ0LCJsZWZ0Ijo2Ljk4MDM5MjQ1NjA1NDY4NzUsIndpZHRoIjo4MS43NTc1MTQ5NTM2MTMyOCwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjY1LjA0OTc0MzY1MjM0Mzc1LCJsZWZ0Ijo2Ljk4MDM5MjQ1NjA1NDY4NzUsIndpZHRoIjo4MS43NTk0NzU3MDgwMDc4MSwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjY4LjUzNDU5MTY3NDgwNDY5LCJsZWZ0Ijo2Ljk4MDM5MjQ1NjA1NDY4NzUsIndpZHRoIjo4MS43MDM5MzM3MTU4MjAzMSwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjcyLjAxOTQ1NDk1NjA1NDY5LCJsZWZ0Ijo2Ljk4MDM5MjQ1NjA1NDY4NzUsIndpZHRoIjo4MS43NTA2NTYxMjc5Mjk2OSwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjc1LjUwODA4NzE1ODIwMzEyLCJsZWZ0Ijo2Ljk4MDM5MjQ1NjA1NDY4NzUsIndpZHRoIjo4MS43NTc1MTQ5NTM2MTMyOCwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjc4Ljk5MjkzNTE4MDY2NDA2LCJsZWZ0IjoxMS43Njg2MjgxMjA0MjIzNjMsIndpZHRoIjo3Ni45NzMxOTc5MzcwMTE3MiwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEyNjg4OCJ9fV0%3D. Rooted in millennia-old wisdom, TEK emphasizes harmony with natural cycles, sustainable harvesting, and community stewardship principles eroded by modernization yet vital for adaptive governance (Foulon et al., 2019)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IsOJdGllbm5lIiwiZmFtaWx5IjoiRm91bG9uIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFsYWluIE4uIiwiZmFtaWx5IjoiUm91c3NlYXUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiR2xlbm4iLCJmYW1pbHkiOiJCZW5veSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJSZWJlY2NhIEwuIiwiZmFtaWx5IjoiTm9ydGgifX1dLCJpZHMiOnsiaXNzbiI6WyIxMjAxLTMwODAiLCIyNDA4LTk0NDMiXSwiaXNibiI6W10sImRvaSI6IjEwLjIxNjYvd3Fyai4yMDE5LjAxMyIsIm1hZyI6IjI5NzM4NjAyODEiLCJvcGVuYWxleCI6IlcyOTczODYwMjgxIn0sInRpdGxlIjoiQSBnbG9iYWwgc2NhbiBvZiBob3cgdGhlIGlzc3VlIG9mIG51dHJpZW50IGxvYWRpbmcgYW5kIGhhcm1mdWwgYWxnYWwgYmxvb21zIGlzIGJlaW5nIGFkZHJlc3NlZCBieSBnb3Zlcm5tZW50cywgbm9uLWdvdmVybm1lbnRhbCBvcmdhbml6YXRpb25zLCBhbmQgdm9sdW50ZWVycyIsImFic3RyYWN0IjoiQWJzdHJhY3QgSGFybWZ1bCBhbGdhbCBibG9vbXMgKEhBQnMpIGluIGFxdWF0aWMgZWNvc3lzdGVtcyBhcmUgb2YgY29uY2VybiB3b3JsZHdpZGUuIFRoaXMgcmV2aWV3IGRlYWxzIHdpdGggaG93IGp1cmlzZGljdGlvbnMgYXJvdW5kIHRoZSB3b3JsZCBhcmUgYWRkcmVzc2luZyB0aGlzIHdhdGVyIHF1YWxpdHkgaXNzdWUgdG8gaW5mb3JtIHJlY29tbWVuZGF0aW9ucyByZWdhcmRpbmcgbnV0cmllbnQgbG9hZGluZyBhbmQgSEFCcyBpbiBNaXNzaXNxdW9pIEJheS1MYWtlIENoYW1wbGFpbiBhbmQgTGFrZSBNZW1waHJlbWFnb2c7IHRyYW5zYm91bmRhcnkgbGFrZXMgbG9jYXRlZCBpbiB0aGUgVVNBIGFuZCBDYW5hZGEgdGhhdCBzdWZmZXIgZnJvbSBzeW1wdG9tcyBvZiBldXRyb3BoaWNhdGlvbi4gQSBnbG9iYWwgc2NhbiBvZiB0aGUgbGl0ZXJhdHVyZSByZXN1bHRlZCBpbiB0aGUgY29uc2lkZXJhdGlvbiBvZiAxMiBjYXNlIHN0dWRpZXMgb2YgbGFyZ2Ugd2F0ZXIgYm9kaWVzIHdpdGhpbiBsYXJnZSB3YXRlcnNoZWRzLCBleGNsdWRpbmcgaW4tbGFrZSBnZW9lbmdpbmVlcmluZyBhcHByb2FjaGVzLiBBbHRob3VnaCBhbGwgb2YgdGhlIHN5c3RlbXMgZXhwZXJpZW5jZSBleGNlc3NpdmUgbnV0cmllbnQgbG9hZGluZywgdGhleSB2YXJ5IGluIHR3byBrZXkgd2F5czogc291cmNlcyBvZiBudXRyaWVudHMgYW5kIG1hbmlmZXN0YXRpb25zIG9mIGV1dHJvcGhpY2F0aW9uIHJhbmdpbmcgZnJvbSBIQUJzLCB0byBsaW1pdGVkIHJlY3JlYXRpb25hbCB1c2VzLCB0byB0aGUgYWRkaXRpb25hbCBjb21wbGV4aXR5IG9mIGludGVybmFsIGxvYWRpbmdzIGFuZCBmaXNoIGtpbGxzLCB1cCB0byBkcmlua2luZyB3YXRlciBzaHV0ZG93bnMuIFRoZSBjYXNlIHN0dWRpZXMgd2VyZSBhbmFseXplZCB3aXRoIHJlc3BlY3QgdG8gZm91ciBjYXRlZ29yaWVzIG9mIGFwcHJvYWNoZXMsIG5hbWVseTogKGkpIHJlZ3VsYXRvcnk7IChpaSkgaW5jZW50aXZlLWJhc2VkOyAoaWlpKSByaXNrIG1pdGlnYXRpb247IGFuZCAoaXYpIG91dHJlYWNoLCBlbmdhZ2VtZW50LCBhbmQgZWR1Y2F0aW9uYWwuIFdlIGZvdW5kIHRoYXQgdGhlIG1hbmFnZW1lbnQgZnJhbWV3b3JrcyBhcmUgYmFzZWQgb24gaW50ZWdyYXRlZCB3YXRlcnNoZWQgbWFuYWdlbWVudCBwbGFubmluZyBhbmQgbmF0aW9uYWwgc3RhbmRhcmRzLiBOYXRpb25hbCB3YXRlciBxdWFsaXR5IHN0YW5kYXJkcywgaG93ZXZlciwgYXJlIG5vdCBzdHJpbmdlbnQgZW5vdWdoIHRvIHByZXZlbnQgSEFCcy4gT3ZlcmFsbCwgaWRlbnRpZmllZCBjYXNlIHN0dWRpZXMgZGlkIG5vdCBzdWNjZXNzZnVsbHkgcmVtZWRpYXRlIEhBQnMsIHRoZXkgc2ltcGx5IG1hbmFnZWQgdGhlbS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjE2Ni93cXJqLjIwMTkuMDEzIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2l3YXBvbmxpbmUuY29tL3dxcmovYXJ0aWNsZS1wZGYvNTUvMS8xLzcwOTI2MS93cXJqYzA1NTAwMDEucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMTktMDktMTciLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA5IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJXYXRlciBRdWFsaXR5IFJlc2VhcmNoIEpvdXJuYWwiLCJpc3N1ZSI6MSwidm9sdW1lIjo1NSwiaW1wYWN0RmFjdG9yIjoiNy44MyJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjEsImxhc3RQYWdlIjoyM319LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJVV0EgUHVibGlzaGluZyJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6ODQsImNpdGVkQnlDb3VudCI6MjB9fSwicXVvdGUiOiJXZSBwcm9wb3NlIHNvbHV0aW9ucyBhbmQgcGF0aHMgZm9yd2FyZCB0byBhIHdpY2tlZCBwcm9ibGVtIHdoaWNoIHNob3VsZCBwcm92ZSBpbnNpZ2h0ZnVsIGZvciBwb2xpY3ktbWFrZXJzIGFuZCBtYW4tIGFnZXJzIGFsaWtlIHRocm91Z2ggdGhlIGZvbGxvd2luZyBmaW5kaW5ncy4gTmF0aW9uYWwgd2F0ZXIgcXVhbGl0eSBzdGFuZGFyZHMgaGF2ZSBub3QgcHJvdmVkIHN0cmluZ2VudCBlbm91Z2ggdG8gcHJlLSB2ZW50IEhBQnMuIElkZW50aWZpZWQgZnJhbWV3b3JrcyBhcmUgYWxsIGJhc2VkIG9uIGludGVncmF0ZWQgd2F0ZXJzaGVkIG1hbmFnZW1lbnQgYW5kIG5hdGlvbmFsIHN0YW5kYXJkcy4gTm8gYWN0dWFsIHN1Y2Nlc3Mgc3RvcmllcyByZWxhdGVkIHRvIHJlbWVkaWF0aW9uIG9mIEhBQnMgd2VyZSBpZGVudGlmaWVkOyB0aGV5IHdlcmUgc2ltcGx5IG1hbmFnZW1lbnQgc3Rvcmllcy4gVGhlIGVmZmljYWN5IG9mIE5QUyBCTVBzIGlzIHF1ZXN0aW9uZWQgdGhyb3VnaG91dCB0aGUgd29ybGQgZHVlIHRvIGluc3VmZmljaWVudCBldmlkZW5jZSB0byBkZXRlcm1pbmUgd2hldGhlciB0aGV5IGNhbiBtaXRpZ2F0ZSBIQUJzLiBNYXJrZXQtYmFzZWQgc3RyYXRlZ2llcyB0byBvZmZzZXQgcG9sbHV0aW9uIG9yIHByb21vdGUgdGFyZ2V0ZWQgQk1QIGFkb3B0aW9uIGFyZSBiZWNvbWluZyBwb3B1bGFyIGFuZCByZXN1bHRpbmcgaW4gY29zdC1lZmZlY3RpdmUgYW5kIGVmZmljaWVudCBhcHByb2FjaGVzLiBDb3N04oCTYmVuZWZpdCBhbmFseXNlcyBtYXkgaW5jcmVhc2UgdGhlIHVzZSBvZiB0ZWNobmljYWwgc29sLSB1dGlvbnMgb3ZlciBjb21wcmVoZW5zaXZlIGFwcHJvYWNoZXMuIEF0IHRoaXMgcG9pbnQsIHRoZXJlIGlzIG5vdCByZWFsbHkgYSB1bmlxdWUgY2FzZSBzdHVkeSB0aGF0IGNvdWxkIGJlIGFwcGxpZWQgdG8gTGFrZSBDaGFtcGxhaW4gYW5kIExha2UgTWVtcGhyZW1hZ29nLiBTcGVjaWZpYyBlbGVtZW50cyBvZiB0aGUgYXBwcm9hY2hlcyBpZGVudGlmaWVkIGluIHRoaXMgcmV2aWV3IGFuZCB1bmktIGZ5aW5nIHRoZW1lcywgaG93ZXZlciwgaG9sZCBwcm9taXNlcy4gVGhpcyBpcyB3aHkgd2UgYWR2b2NhdGUgZm9yIGNvbmR1Y3RpbmcgYSBzeXN0ZW1hdGljIGxpdGVyYXR1cmUgcmV2aWV3IHdpdGggcmVnYXJkIHRvIHRob3NlIHRoZW1lcyBpbiBvcmRlciB0byBkZXZlbG9wIHJlY29tbWVuLSBkYXRpb25zIHRhaWxvcmVkIHRvIHRoZSBMYWtlIE1lbXBocmVtYWdvZyBhbmQgTWlzc2lzcXVvaSBCYXktTGFrZSBDaGFtcGxhaW4gYXJlYXMuIEF0IGxhc3QsIHdlIHdvdWxkIGxpa2UgdG8gYWRkcmVzcyB0aGUgZWxlcGhhbnQtc2l6ZWQgcXVlcy0gdGlvbiBpbiB0aGUgcm9vbS4gV2lsbCBjbGltYXRlIGNoYW5nZSBpbmNyZWFzZSBIQUJzIGZyZXF1ZW5jeT8gSW50ZXJhY3RpdmUgZWZmZWN0cyBvZiBldXRyb3BoaWNhdGlvbiBhbmQgY2xpbWF0ZSBjaGFuZ2Ugb24gaGFybWZ1bCBjeWFub2JhY3RlcmlhbCBibG9vbXMgYXJlIGNvbXBsZXguICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxNSwiYmJveCI6eyJ0b3AiOjg1LjYwNTUxNDUyNjM2NzE5LCJsZWZ0Ijo0OS43OTQ4NzIyODM5MzU1NSwid2lkdGgiOjM5LjcxODY2MjI2MTk2Mjg5LCJoZWlnaHQiOjEuMzI4NjcxMzM2MTc0MDExMn19LHsicGFnZU51bWJlciI6MTUsImJib3giOnsidG9wIjo4Ny4zOTUzNjI4NTQwMDM5LCJsZWZ0Ijo0OS43OTQ4NzIyODM5MzU1NSwid2lkdGgiOjM5LjcwMjcwOTE5Nzk5ODA1LCJoZWlnaHQiOjEuMzI4NjcxMzM2MTc0MDExMn19LHsicGFnZU51bWJlciI6MTUsImJib3giOnsidG9wIjo4OS4xODUyMTExODE2NDA2MiwibGVmdCI6NDkuNzk0ODcyMjgzOTM1NTUsIndpZHRoIjozOS43MDIwOTUwMzE3MzgyOCwiaGVpZ2h0IjoxLjMyODY3MTMzNjE3NDAxMTJ9fSx7InBhZ2VOdW1iZXIiOjE1LCJiYm94Ijp7InRvcCI6OTAuOTc1MDU5NTA5Mjc3MzQsImxlZnQiOjQ5Ljc5NDg3MjI4MzkzNTU1LCJ3aWR0aCI6MzkuNzA0NjIwMzYxMzI4MTI1LCJoZWlnaHQiOjEuMzI4NjcxMzM2MTc0MDExMn19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjoxNC4wMjU5OTA0ODYxNDUwMiwibGVmdCI6MTAuNTAyMzIzMTUwNjM0NzY2LCJ3aWR0aCI6MzkuNzA1OTU1NTA1MzcxMDk0LCJoZWlnaHQiOjEuMzI4NjY5NDI4ODI1Mzc4NH19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjoxNS44MTU4NDE2NzQ4MDQ2ODgsImxlZnQiOjEwLjUwMjMyMzE1MDYzNDc2Niwid2lkdGgiOjM5LjcwMjI0NzYxOTYyODkwNiwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MTcuNjA1NjkxOTA5NzkwMDQsImxlZnQiOjEwLjUwMjE3NjI4NDc5MDAzOSwid2lkdGgiOjM5LjcwMDkyMzkxOTY3NzczNCwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MTkuMzk1NTQ0MDUyMTI0MDIzLCJsZWZ0IjoxMC41MDIxNzYyODQ3OTAwMzksIndpZHRoIjozOS43MDg5MDA0NTE2NjAxNTYsImhlaWdodCI6MS4zMjg2Njk0Mjg4MjUzNzg0fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjIxLjE4NTM5NDI4NzEwOTM3NSwibGVmdCI6MTAuNTAyMTc2Mjg0NzkwMDM5LCJ3aWR0aCI6MzkuNzEwMDgzMDA3ODEyNSwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MjIuOTc1MjQ0NTIyMDk0NzI3LCJsZWZ0IjoxMC41MDIxNzYyODQ3OTAwMzksIndpZHRoIjozOS43MDQwMjkwODMyNTE5NSwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MjQuNzY1MDk2NjY0NDI4NzEsImxlZnQiOjEwLjUwMjE3NjI4NDc5MDAzOSwid2lkdGgiOjM5LjcwMzg4MDMxMDA1ODU5NCwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MjYuNTU0OTQ4ODA2NzYyNjk1LCJsZWZ0IjoxMC41MDIwMjc1MTE1OTY2OCwid2lkdGgiOjM5LjcwMzg4MDMxMDA1ODU5NCwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MjguMzQ0Nzk5MDQxNzQ4MDQ3LCJsZWZ0IjoxMC41MDIwMjc1MTE1OTY2OCwid2lkdGgiOjM5LjcwMDAzODkwOTkxMjExLCJoZWlnaHQiOjEuMzI4NjY5NDI4ODI1Mzc4NH19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjozMC4xMzQ2NDczNjkzODQ3NjYsImxlZnQiOjEwLjUwMjAyNzUxMTU5NjY4LCJ3aWR0aCI6MzkuNzA0NjE2NTQ2NjMwODYsImhlaWdodCI6MS4zMjg2Njk0Mjg4MjUzNzg0fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjMxLjkyNDQ5NzYwNDM3MDExNywibGVmdCI6MTAuNTAxODc3Nzg0NzI5MDA0LCJ3aWR0aCI6MzkuNzA1Nzg3NjU4NjkxNDA2LCJoZWlnaHQiOjEuMzI4NjY5NDI4ODI1Mzc4NH19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjozMy43MTQzNTE2NTQwNTI3MzQsImxlZnQiOjEwLjUwMTcyOTAxMTUzNTY0NSwid2lkdGgiOjM5LjcxOTI0NTkxMDY0NDUzLCJoZWlnaHQiOjEuMzI4NjY3NDAyMjY3NDU2fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjM1LjUwNDIwMzc5NjM4NjcyLCJsZWZ0IjoxMC41MDE3MjkwMTE1MzU2NDUsIndpZHRoIjozOS43MDI3MDkxOTc5OTgwNSwiaGVpZ2h0IjoxLjMyODY2NzQwMjI2NzQ1Nn19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjozNy4yOTQwNTIxMjQwMjM0NCwibGVmdCI6MTAuNTAxNzI5MDExNTM1NjQ1LCJ3aWR0aCI6MzkuNzA1Nzk5MTAyNzgzMiwiaGVpZ2h0IjoxLjMyODY2NzQwMjI2NzQ1Nn19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjozOS4wODM5MDQyNjYzNTc0MiwibGVmdCI6MTAuNTAxNzI1MTk2ODM4Mzc5LCJ3aWR0aCI6MzkuNzA2NTM1MzM5MzU1NDcsImhlaWdodCI6MS4zMjg2Njc0MDIyNjc0NTZ9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6NDAuODczNzU2NDA4NjkxNDA2LCJsZWZ0IjoxMC41MDE3MjcxMDQxODcwMTIsIndpZHRoIjozOS43MjExNTMyNTkyNzczNDQsImhlaWdodCI6MS4zMjg2Njc0MDIyNjc0NTZ9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6NDIuNjYzNjA4NTUxMDI1MzksImxlZnQiOjEwLjUwMTcyNzEwNDE4NzAxMiwid2lkdGgiOjM5LjcwMzEzNjQ0NDA5MTgsImhlaWdodCI6MS4zMjg2Njc0MDIyNjc0NTZ9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6NDQuNDQ2MjA4OTUzODU3NDIsImxlZnQiOjEwLjUwMTcyNzEwNDE4NzAxMiwid2lkdGgiOjM5LjcwMDc3NTE0NjQ4NDM3NSwiaGVpZ2h0IjoxLjMyODY2NzQwMjI2NzQ1Nn19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjo0Ni4yMzYwNjEwOTYxOTE0MDYsImxlZnQiOjEwLjUwMTcyNzEwNDE4NzAxMiwid2lkdGgiOjM5LjcwNzcyNTUyNDkwMjM0NCwiaGVpZ2h0IjoxLjMyODY2NzQwMjI2NzQ1Nn19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjo0OC4wMjU5MTMyMzg1MjUzOSwibGVmdCI6MTAuNTAxNzI3MTA0MTg3MDEyLCJ3aWR0aCI6MjUuOTQxNjY3NTU2NzYyNjk1LCJoZWlnaHQiOjEuMzI4NjY3NDAyMjY3NDU2fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjQ5LjgxNTc2MTU2NjE2MjExLCJsZWZ0IjoxMy4xMjUwMTYyMTI0NjMzNzksIndpZHRoIjozNy4wODExODA1NzI1MDk3NjYsImhlaWdodCI6MS4zMjg2Njc0MDIyNjc0NTZ9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6NTEuNjA1NjEzNzA4NDk2MDk0LCJsZWZ0IjoxMC41MDE3MjcxMDQxODcwMTIsIndpZHRoIjozOS43MDkyMDU2Mjc0NDE0MDYsImhlaWdodCI6MS4zMjg2Njc0MDIyNjc0NTZ9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6NTMuMzk1NDYyMDM2MTMyODEsImxlZnQiOjEwLjUwMTcyNzEwNDE4NzAxMiwid2lkdGgiOjM5LjcxOTIyMzAyMjQ2MDk0LCJoZWlnaHQiOjEuMzI4NjY3NDAyMjY3NDU2fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjU1LjE4NTMxMDM2Mzc2OTUzLCJsZWZ0IjoxMC41MDE3MjcxMDQxODcwMTIsIndpZHRoIjozOS43MTA2NzgxMDA1ODU5NCwiaGVpZ2h0IjoxLjMyODY2NzQwMjI2NzQ1Nn19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjE2In19XQ%3D%3D. Historical practices like rainwater harvesting through johads (earthen check dams) in arid Rajasthan and eris (temple tanks) in Tamil Nadu demonstrate low-tech, decentralized systems that recharge groundwater, dilute pollutants, and foster biodiversity, countering nutrient overload from urbanization (Berdalet et al., 2017)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkVsaXNhIiwiZmFtaWx5IjoiQmVyZGFsZXQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUmFwaGFlbCBNLiIsImZhbWlseSI6Ikt1ZGVsYSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJFZCIsImZhbWlseSI6IlVyYmFuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhlbnJpayIsImZhbWlseSI6IkVuZXZvbGRzZW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTmVpbCBTLiIsImZhbWlseSI6IkJhbmFzIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkVpbGVlbiIsImZhbWlseSI6IkJyZXNuYW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTWljaGVsZSBBLiIsImZhbWlseSI6IkJ1cmZvcmQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiS2VpdGgiLCJmYW1pbHkiOiJEYXZpZHNvbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJDaHJpc3RvcGhlciBKLiIsImZhbWlseSI6IkdvYmxlciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJCZW5ndCIsImZhbWlseSI6Ikthcmxzb24ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUG8gVGVlbiIsImZhbWlseSI6IkxpbSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJMaW5jb2xuIiwiZmFtaWx5IjoiTWFjS2VuemllIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1hcmluYSIsImZhbWlseSI6Ik1vbnRyZXNvciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJWZXJhIEwuIiwiZmFtaWx5IjoiVHJhaW5lciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJHaXJlcyIsImZhbWlseSI6IlVzdXAifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiS2Vkb25nIiwiZmFtaWx5IjoiWWluIn19XSwiaWRzIjp7Imlzc24iOlsiMTA0Mi04Mjc1IiwiMjM3Ny02MTdYIl0sImlzYm4iOltdLCJkb2kiOiIxMC41NjcwL29jZWFub2cuMjAxNy4xMTEiLCJtYWciOiIyNTk3NjY0NzI3Iiwib3BlbmFsZXgiOiJXMjU5NzY2NDcyNyJ9LCJ0aXRsZSI6Ikdsb2JhbEhBQjogQSBOZXcgUHJvZ3JhbSB0byBQcm9tb3RlIEludGVybmF0aW9uYWwgUmVzZWFyY2gsIE9ic2VydmF0aW9ucywgYW5kIE1vZGVsaW5nIG9mIEhhcm1mdWwgQWxnYWwgQmxvb21zIGluIEFxdWF0aWMgU3lzdGVtcyIsImFic3RyYWN0IjoiQmVyZGFsZXQsIEVsaXNhIC4uLiBldCBhbC4tLSBTcGVjaWFsIGlzc3VlIG9uIEludGVybmF0aW9uYWwgQ29vcGVyYXRpb24gaW4gSGFybWZ1bCBBbGdhbCBCbG9vbSBTY2llbmNlLi0tIDEyIHBhZ2VzLCA2IGZpZ3VyZXMiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuNTY3MC9vY2Vhbm9nLjIwMTcuMTExIiwicmVzb3VyY2VVcmwiOiJodHRwczovL3Rvcy5vcmcvb2NlYW5vZ3JhcGh5L2Fzc2V0cy9kb2NzLzMwLTFfYmVyZGFsZXQzLnBkZiIsImlzc3VlZERhdGUiOiIyMDE3LTAzLTAxIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiT2NlYW5vZ3JhcGh5IiwiaXNzdWUiOjEsInZvbHVtZSI6MzAsImltcGFjdEZhY3RvciI6IjIuOTMifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjo3MCwibGFzdFBhZ2UiOjgxfX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6Ik9jZWFub2dyYXBoeSBTb2NpZXR5In0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjoyNiwiY2l0ZWRCeUNvdW50IjozMn19LCJxdW90ZSI6IlRoZSAgZWNvbm9taWMgZWZmZWN0cyBvZiBIQUJzIGFyaXNlIGZyb20gcHViLWxpYyBoZWFsdGggY29zdHMsIGNvbW1lcmNpYWwgZmlzaGVyeSBhbmQgIGFxdWFjdWx0dXJlIG9wZXJhdGlvbiBjbG9zdXJlcyBhbmQgZmlzaCAga2lsbHMsIHBvc3NpYmxlIG1lZGl1bS0gYW5kIGxvbmctdGVybSAgZGVjbGluZXMgaW4gY29hc3RhbCBhbmQgbWFyaW5lIHJlY3JlYXRpb24gIGFuZCB0b3VyaXNtLCBhbmQgdGhlIGNvc3RzIG9mIGluc3VyYW5jZSwgIG1vbml0b3JpbmcsIG1hbmFnZW1lbnQsIGFuZCBtaXRpZ2F0aW9uICAoTW9yZ2FuIGV0IGFsLiwgMjAxMCkuIEFnZ3JlZ2F0aW5nIGVjby1ub21pYyBlZmZlY3Rz4oCUYm90aCB3aXRoaW4gYW5kIGFjcm9zcyAgdGhlc2UgY2F0ZWdvcmllc%2BKAlGNhbiBhbHNvIGJlIHByb2JsZW0tYXRpYyB3aXRoIGF2YWlsYWJsZSBkYXRhIGFuZCBjdXJyZW50bHkgdXNlZCAgbWV0aG9kb2xvZ2llcyAoSG9hZ2xhbmQgZXQgYWwuLCAyMDAyKS4gIEFuIGV4YW1wbGUgb2YgYW4gZWNvbm9taWMgZXZhbHVhdGlvbiAgb2YgSEFCcyBhdCBhIG5hdGlvbmFsIGxldmVsIGlzIGdpdmVuIGJ5ICBBbmRlcnNvbiBldCBhbC4gKDIwMDApLCB3aG8gZXN0aW1hdGVkIHRoZSAgZWNvbm9taWMgZWZmZWN0cyBvZiBIQUJzIGluIHRoZSBVbml0ZWQgIFN0YXRlcyB0byBiZSAkMTAwIG1pbGxpb24gcGVyIHllYXIgKGFzIGxhdGVyICB0cmFuc2xhdGVkIGludG8gMjAxMiBkb2xsYXJzKSwgYnJva2VuIGRvd24gIGFzIGZvbGxvd3M6IDQ1JSBmb3IgcHVibGljIGhlYWx0aCBjb3N0cywgIDM3JSBmb3IgdGhlIGNvc3RzIG9mIGNsb3N1cmVzIGFuZCBsb3NzZXMgIGV4cGVyaWVuY2VkIGJ5IGNvbW1lcmNpYWwgZmlzaGVyaWVzLCAgMTMlIGZyb20gdGhlIGltcGFjdCBvbiBsb3N0IHJlY3JlYXRpb24gIGFuZCB0b3VyaXNtIG9wcG9ydHVuaXRpZXMsIGFuZCA0JSBmcm9tIEZJR1VSRSA2LiBNYXNzaXZlIGZpc2ggbW9ydGFsaXRpZXMgaW4gSmFrYXJ0YSwgSW5kb25lc2lhLiBQaG90byBjcmVkaXRzOiBUdW1wYWsgU2lkYWJ1dGFyIChIQU41MykgbW9uaXRvcmluZyBhbmQgbWFuYWdlbWVudCBjb3N0cy4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjoxMS44MTk0NDQ2NTYzNzIwNywibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzQ5NDUyOTcyNDEyMSwiaGVpZ2h0IjoxLjcwMzI4MjU5NDY4MDc4NjF9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjoxMy41MjM5ODk2Nzc0MjkyLCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI2Ljg2NDM3NDE2MDc2NjYsImhlaWdodCI6MS43MDMyODI1OTQ2ODA3ODYxfX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6MTUuMjI4NTM1NjUyMTYwNjQ1LCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzMjAwMjI1ODMwMDc4LCJoZWlnaHQiOjEuNzAzMjgzNTQ4MzU1MTAyNX19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjE2LjkzMzA4MDY3MzIxNzc3MywibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzM2MzY4NTYwNzkxLCJoZWlnaHQiOjEuNzAzMjgzNTQ4MzU1MTAyNX19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjE4LjYzNzYyNjY0Nzk0OTIyLCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzNTI5NDM0MjA0MTAxNiwiaGVpZ2h0IjoxLjcwMzI4MzU0ODM1NTEwMjV9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjoyMC4zNDIxNzA3MTUzMzIwMywibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzM5ODU5MDA4Nzg5MDYsImhlaWdodCI6MS43MDMyODM1NDgzNTUxMDI1fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6MjIuMDQ2NzE2NjkwMDYzNDc3LCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzMjAxMTc5NTA0Mzk0NSwiaGVpZ2h0IjoxLjcwMzI4MzU0ODM1NTEwMjV9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjoyMy43NTEyNjI2NjQ3OTQ5MjIsImxlZnQiOjY3LjI1NDg5ODA3MTI4OTA2LCJ3aWR0aCI6MjcuMjMyMDQyMzEyNjIyMDcsImhlaWdodCI6MS43MDMyODM1NDgzNTUxMDI1fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6MjUuNDU1ODA4NjM5NTI2MzY3LCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI2Ljg2MzA2NzYyNjk1MzEyNSwiaGVpZ2h0IjoxLjcwMzI4MzU0ODM1NTEwMjV9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjoyNy4xNjAzNTI3MDY5MDkxOCwibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzU0MzM1Nzg0OTEyMSwiaGVpZ2h0IjoxLjcwMzI4MzU0ODM1NTEwMjV9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjoyOC44NjQ4OTg2ODE2NDA2MjUsImxlZnQiOjY3LjI1NDg5ODA3MTI4OTA2LCJ3aWR0aCI6MjYuODY0Mzk1MTQxNjAxNTYyLCJoZWlnaHQiOjEuNzAzMjgzNTQ4MzU1MTAyNX19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjMwLjU2OTQ0Mjc0OTAyMzQzOCwibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzQ5MjQzMTY0MDYyNSwiaGVpZ2h0IjoxLjcwMzI4MzU0ODM1NTEwMjV9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjozMi4yNzM5OTA2MzExMDM1MTYsImxlZnQiOjY3LjI1NDg5ODA3MTI4OTA2LCJ3aWR0aCI6MjcuMjM0Nzc1NTQzMjEyODksImhlaWdodCI6MS43MDMyODM1NDgzNTUxMDI1fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6MzMuOTc4NTM0Njk4NDg2MzMsImxlZnQiOjY3LjI1NDg5ODA3MTI4OTA2LCJ3aWR0aCI6MjcuMjM0MzM2ODUzMDI3MzQ0LCJoZWlnaHQiOjEuNzAzMjg1NDU1NzAzNzM1NH19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjM1LjY4MzA3ODc2NTg2OTE0LCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzMjE2MDU2ODIzNzMwNSwiaGVpZ2h0IjoxLjcwMzI4NTQ1NTcwMzczNTR9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjozNy4zODc2MjI4MzMyNTE5NSwibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzM2NjU0NjYzMDg1OTQsImhlaWdodCI6MS43MDMyODU0NTU3MDM3MzU0fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6MzkuMDkyMTcwNzE1MzMyMDMsImxlZnQiOjY3LjI1NDg5ODA3MTI4OTA2LCJ3aWR0aCI6MjcuMjM2NTc5ODk1MDE5NTMsImhlaWdodCI6MS43MDMyODU0NTU3MDM3MzU0fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6NDAuNzk2NzE0NzgyNzE0ODQ0LCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzNjcxNzIyNDEyMTA5NCwiaGVpZ2h0IjoxLjcwMzI4NTQ1NTcwMzczNTR9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo0Mi41MDEyNjI2NjQ3OTQ5MiwibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzM2MjczMTkzMzU5MzgsImhlaWdodCI6MS43MDMyODU0NTU3MDM3MzU0fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6NDQuMjA1ODA2NzMyMTc3NzM0LCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzMDU0NTA0Mzk0NTMxMiwiaGVpZ2h0IjoxLjcwMzI4NTQ1NTcwMzczNTR9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo0NS45MTAzNTA3OTk1NjA1NSwibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzg1NDQ0NjQxMTEzMjgsImhlaWdodCI6MS43MDMyODU0NTU3MDM3MzU0fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6NDcuNjE0ODk0ODY2OTQzMzYsImxlZnQiOjY3LjI1NDg5ODA3MTI4OTA2LCJ3aWR0aCI6MjcuMjM0MTQ2MTE4MTY0MDYyLCJoZWlnaHQiOjEuNzAzMjg1NDU1NzAzNzM1NH19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjQ5LjMxOTQ0Mjc0OTAyMzQ0LCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzNTk0MDkzMzIyNzU0LCJoZWlnaHQiOjEuNzAzMjg1NDU1NzAzNzM1NH19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjUxLjAyMzk5MDYzMTEwMzUxNiwibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzY5MDc5NTg5ODQzNzUsImhlaWdodCI6MS43MDMyODU0NTU3MDM3MzU0fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6OTIuMjc4NzcwNDQ2Nzc3MzQsImxlZnQiOjguODIzNTI5MjQzNDY5MjM4LCJ3aWR0aCI6NTYuMjYzOTE2MDE1NjI1LCJoZWlnaHQiOjEuMjA5MDg5NTE3NTkzMzgzOH19LHsicGFnZU51bWJlciI6OSwiYmJveCI6eyJ0b3AiOjUuMDAxMjYyNjY0Nzk0OTIyLCJsZWZ0Ijo1Ljg4MjM1MjgyODk3OTQ5Miwid2lkdGgiOjI3LjIzNTI5NDM0MjA0MTAxNiwiaGVpZ2h0IjoxLjcwMzI4MjU5NDY4MDc4NjF9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI3OCJ9fV0%3D. Unlike rigid BMPs outpaced by warming-induced stratification, these methods integrate socio-ecological feedbacks, reducing internal nutrient loading by mimicking pre-industrial hydrology (Meerhoff et al., 2022; Paerl et al., 2019)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik0uIiwiZmFtaWx5IjoiTWVlcmhvZmYifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSi4iLCJmYW1pbHkiOiJBdWRldCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJULkEuIiwiZmFtaWx5IjoiRGF2aWRzb24ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTC4gRGUiLCJmYW1pbHkiOiJNZWVzdGVyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlMuIiwiZmFtaWx5IjoiSGlsdCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTLiIsImZhbWlseSI6Iktvc3RlbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJaYWlodWEiLCJmYW1pbHkiOiJMaXUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTi4iLCJmYW1pbHkiOiJNYXp6ZW8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSGFucyBXLiIsImZhbWlseSI6IlBhZXJsIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik0uIiwiZmFtaWx5IjoiU2NoZWZmZXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiRS4iLCJmYW1pbHkiOiJKZXBwZXNlbiJ9fV0sImlkcyI6eyJpc3NuIjpbXSwiaXNibiI6W10sImRvaSI6IjEwLjE3NjE1Lzd2ZTUtdzAxNiIsIm9wZW5hbGV4IjoiVzQzMTg0MTU1NzcifSwidGl0bGUiOiJGZWVkYmFjayBiZXR3ZWVuIGNsaW1hdGUgY2hhbmdlIGFuZCBldXRyb3BoaWNhdGlvbjogcmV2aXNpdGluZyB0aGUgYWxsaWVkIGF0dGFjayBjb25jZXB0IGFuZCBob3cgdG8gc3RyaWtlIGJhY2siLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTc2MTUvN3ZlNS13MDE2IiwicmVzb3VyY2VVcmwiOiJodHRwczovL2Nkci5saWIudW5jLmVkdS9kb3dubG9hZHMvaDk4OXJjOTM4IiwiaXNzdWVkRGF0ZSI6IjIwMjItMDEtMDEiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTEwIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJDYXJvbGluYSBEaWdpdGFsIFJlcG9zaXRvcnkgKFVuaXZlcnNpdHkgb2YgTm9ydGggQ2Fyb2xpbmEgYXQgQ2hhcGVsIEhpbGwpIiwiaW1wYWN0RmFjdG9yIjoiMC4xMCJ9LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJVbml2ZXJzaXR5IG9mIE5vcnRoIENhcm9saW5hIGF0IENoYXBlbCBIaWxsIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjowLCJjaXRlZEJ5Q291bnQiOjN9fSwicXVvdGUiOiJXaXRoIGNsaW1hdGUgd2FybWluZywgdGhlIGRpcmVjdCBhbmQgaW5kaS1yZWN0IGVmZmVjdHMgb2YgaW5jcmVhc2VkIGxha2UgdGVtcGVyYXR1cmVzIHdpbGwgbGlrZWx5IGRyaXZlIGNoYW5nZXMgaW4gY3lhbm9iYWN0ZXJpYWwgdGF4YSBhbmQgdGhlaXIgcHJvZHVjLXRpb24gb2YgY3lhbm9iYWN0ZXJpYWwgdG94aW5zLCBsaWtlbHkgaW5jbHVkaW5nIHByb21vdC1pbmcgdGhlIHNlbGVjdGlvbiBvZiBhIGZldyBNaWNyb2N5c3RpcyBzcGVjaWVzIG9yIHN0cmFpbnMgcHJvZHVjaW5nIGhpZ2hseSB0b3hpYyBtaWNyb2N5c3Rpbi1MUiAoTWFudHpvdWtpIGV0IGFsLiAyMDE4KS4gRGVzcGl0ZSB2YXJpYXRpb25zIGFtb25nIGxha2VzIGluIGEgd2F0ZXItc2NhcGUsIGEgcmVnaW9uYWwgaW5jcmVhc2UgaW4gbWljcm9jeXN0aW4gbWF4aW1hIGFuZCBkdXJhdGlvbiBvZiBibG9vbXMgaGFzIGFsc28gYmVlbiBjbGVhcmx5IGV4cGxhaW5lZCBieSBpbmNyZWFzaW5nIHRlbXBlcmF0dXJlIGluIGNlbnRyYWwgTm9ydGggQW1lcmljYSAoSGF5ZXMgZXQgYWwuIDIwMjApLCB3aXRoIGluY3JlYXNpbmcgcmlza3MgdG8gaHVtYW4gaGVhbHRoLiBHcm93aW5nIGV2aWRlbmNlIGluZGljYXRlcyB0aGF0IGN5YW5vYmFjdGVyaWFsIGRvbWluYW5jZSB3aWxsIGV4ZXJ0IGEgcG9zaXRpdmUgZmVlZGJhY2sgd2l0aCBjbGltYXRlIHdhcm1pbmcgKEhhdmVucyBldCBhbC4gMjAxNiwgUGFlcmwgZXQgYWwuIDIwMjApLiBNYW55IGN5YW5vYmFjdGVyaWFsIHNwZWNpZXMgcHJvbGlmZXJhdGUgb24gdGhlIGxha2Ugc3VyZmFjZSwgYWJzb3JiaW5nIGxpZ2h0LCBlbmhhbmNpbmcgbG9jYWxpemVkIHRlbXBlcmF0dXJlIGluY3JlYXNlcywgYW5kIGluY3JlYXNpbmcgdGhlcm1hbCBzdHJhdGlmaWNhdGlvbiBwb3RlbnRpYWwgKEthaHJ1IGV0IGFsLiAxOTkzKS4gQ3lhbm9iYWN0ZXJpYSBwcm9tb3RlIHRoZSByZWxlYXNlIG9mIEdIR3MsIHBhcnRpY3VsYXJseSBDSDQgKFlhbiBldCBhbC4gMjAxNyksIGR1cmluZyB0aGUgZGVjb21wb3NpdGlvbiBmb2xsb3dpbmcgdGhlIGNvbGxhcHNlIG9mIGEgYmxvb20uIEN5YW5vYmFjdGVyaWEgYWxzbyBwcm9tb3RlIHJlbWluZXJhbGl6YXRpb24gb2YgbnV0cmktZW50cyBkdXJpbmcgYmxvb21zLCBsZWFkaW5nIHRvIG11bHRpcGxlIGJsb29tcyBpbiBhIHNpbmdsZSBzZWFzb24uIFVuZGVyIGV1dHJvcGhpYyBhbmQgd2FybWluZyBjb25kaS10aW9ucywgY3lhbm9iYWN0ZXJpYSBkb21pbmFuY2UgbWF5IHRodXMgYmUgYSBzZWxmLXBlcnBldHVhdGluZyBwcm9waGVjeSAoRmlnLiAyIGFuZCAzKS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo2Ny4wNjM0MDAyNjg1NTQ2OSwibGVmdCI6NTEuNDc4ODAxNzI3Mjk0OTIsIndpZHRoIjozOC42NzM0MjM3NjcwODk4NDQsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6NjguNjk4NTQ3MzYzMjgxMjUsImxlZnQiOjUxLjQ3ODgwMTcyNzI5NDkyLCJ3aWR0aCI6MzguNjc5NDk2NzY1MTM2NzIsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6NzAuMzMzNjg2ODI4NjEzMjgsImxlZnQiOjUxLjQ3ODgwMTcyNzI5NDkyLCJ3aWR0aCI6MzguNjc3NTgxNzg3MTA5Mzc1LCJoZWlnaHQiOjEuNDcxMTAxNTIyNDQ1Njc4N319LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjcxLjk2ODgyNjI5Mzk0NTMxLCJsZWZ0Ijo1MS40Nzg4MDE3MjcyOTQ5Miwid2lkdGgiOjM4LjY3OTEwMzg1MTMxODM2LCJoZWlnaHQiOjEuNDcxMTAxNTIyNDQ1Njc4N319LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjczLjYxMDU3MjgxNDk0MTQsImxlZnQiOjUxLjQ3ODgwMTcyNzI5NDkyLCJ3aWR0aCI6MzguNjc0NDI3MDMyNDcwNywiaGVpZ2h0IjoxLjQ3Mzc0Njg5NTc5MDF9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo3NS4yNDcwMzk3OTQ5MjE4OCwibGVmdCI6NTEuNDc4ODA5MzU2Njg5NDUsIndpZHRoIjozOC42NzQyNjMwMDA0ODgyOCwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo3Ni44ODIxODY4ODk2NDg0NCwibGVmdCI6NTEuNDc4ODA5MzU2Njg5NDUsIndpZHRoIjozOC42ODExNzE0MTcyMzYzMywiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo3OC41MjUyNTMyOTU4OTg0NCwibGVmdCI6NTEuNDc4ODA5MzU2Njg5NDUsIndpZHRoIjozOC42NzU2NjI5OTQzODQ3NjYsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODAuMTYwNDAwMzkwNjI1LCJsZWZ0Ijo1MS40Nzg4MDkzNTY2ODk0NSwid2lkdGgiOjM4LjY4MjAyOTcyNDEyMTA5NCwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo4MS43OTU1Mzk4NTU5NTcwMywibGVmdCI6NTEuNDc4ODA5MzU2Njg5NDUsIndpZHRoIjozOC42NzQ5ODM5NzgyNzE0ODQsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODMuNDM4NjEzODkxNjAxNTYsImxlZnQiOjUxLjQ3ODgwOTM1NjY4OTQ1LCJ3aWR0aCI6MzguNjc2MTg1NjA3OTEwMTU2LCJoZWlnaHQiOjEuNDg2OTczNTI0MDkzNjI4fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODUuMDg5NjMwMTI2OTUzMTIsImxlZnQiOjUxLjQ3ODgwOTM1NjY4OTQ1LCJ3aWR0aCI6NC42ODI4MDUwNjEzNDAzMzIsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODYuNzI0Nzc3MjIxNjc5NjksImxlZnQiOjUzLjQ0MTE3NzM2ODE2NDA2LCJ3aWR0aCI6Ni4xNDAzNTU1ODcwMDU2MTUsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODYuNzI0ODkxNjYyNTk3NjYsImxlZnQiOjYwLjk5NDg3Njg2MTU3MjI2Niwid2lkdGgiOjYuMDM2OTkyNTQ5ODk2MjQsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODYuNzI0ODkxNjYyNTk3NjYsImxlZnQiOjY4LjQ0NjUyNTU3MzczMDQ3LCJ3aWR0aCI6Ni4xNTA3MDU4MTQzNjE1NzIsImhlaWdodCI6MS40NzExMDE1MjI0NDU2Nzg3fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6ODYuNzI0ODkxNjYyNTk3NjYsImxlZnQiOjc2LjAwOTgyNjY2MDE1NjI1LCJ3aWR0aCI6Mi43MjczMzMzMDcyNjYyMzU0LCJoZWlnaHQiOjEuNDcxMTAxNTIyNDQ1Njc4N319LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjg2LjcyNDg5MTY2MjU5NzY2LCJsZWZ0Ijo4MC4xNDk1ODE5MDkxNzk2OSwid2lkdGgiOjEwLjAwODIzNDk3NzcyMjE2OCwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo4OC4zNjAwMzg3NTczMjQyMiwibGVmdCI6NTEuNDc3NjU3MzE4MTE1MjM0LCJ3aWR0aCI6MzguNjcyMDU0MjkwNzcxNDg0LCJoZWlnaHQiOjEuNDcxMTAxNTIyNDQ1Njc4N319LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjkwLjAwMzExMjc5Mjk2ODc1LCJsZWZ0Ijo1MS40Nzc2NTczMTgxMTUyMzQsIndpZHRoIjozOC42NzY0OTg0MTMwODU5NCwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo5MS42MzgyNTIyNTgzMDA3OCwibGVmdCI6NTEuNDc3NjUzNTAzNDE3OTcsIndpZHRoIjozOC42NzU4MzQ2NTU3NjE3MiwiaGVpZ2h0IjoxLjQ3MTEwMTUyMjQ0NTY3ODd9fSx7InBhZ2VOdW1iZXIiOjksImJib3giOnsidG9wIjo2LjQ2ODkzNTk2NjQ5MTY5OSwibGVmdCI6OS44NDAxMzM2NjY5OTIxODgsIndpZHRoIjozOC42ODYzMjUwNzMyNDIxOSwiaGVpZ2h0IjoxNi4yMTIxMjU3NzgxOTgyNDJ9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI5In19LHsib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhhbnMgVy4iLCJmYW1pbHkiOiJQYWVybCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLYXJsIEUuIiwiZmFtaWx5IjoiSGF2ZW5zIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hdGhhbiBTLiIsImZhbWlseSI6IkhhbGwifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiVGltb3RoeSBHLiIsImZhbWlseSI6Ik90dGVuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1lbmd5dWFuIiwiZmFtaWx5IjoiWmh1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhhaSIsImZhbWlseSI6Ilh1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikd1YW5nd2VpIiwiZmFtaWx5IjoiWmh1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkJvcWlhbmciLCJmYW1pbHkiOiJRaW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIxMzIzLTE2NTAiLCIxNDQ4LTYwNTkiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwNzEvbWYxODM5MiIsIm1hZyI6IjI5NDIzMjE1NDIiLCJvcGVuYWxleCI6IlcyOTQyMzIxNTQyIn0sInRpdGxlIjoiTWl0aWdhdGluZyBhIGdsb2JhbCBleHBhbnNpb24gb2YgdG94aWMgY3lhbm9iYWN0ZXJpYWwgYmxvb21zOiBjb25mb3VuZGluZyBlZmZlY3RzIGFuZCBjaGFsbGVuZ2VzIHBvc2VkIGJ5IGNsaW1hdGUgY2hhbmdlIiwiYWJzdHJhY3QiOiJNYW5hZ2luZyBhbmQgbWl0aWdhdGluZyB0aGUgZ2xvYmFsIGV4cGFuc2lvbiBvZiB0b3hpYyBjeWFub2JhY3RlcmlhbCBoYXJtZnVsIGFsZ2FsIGJsb29tcyAoQ3lhbm9IQUJzKSBpcyBhIG1ham9yIGNoYWxsZW5nZSBmYWNpbmcgcmVzZWFyY2hlcnMgYW5kIHdhdGVyIHJlc291cmNlIG1hbmFnZXJzLiBWYXJpb3VzIGFwcHJvYWNoZXMsIGluY2x1ZGluZyBudXRyaWVudCBsb2FkIHJlZHVjdGlvbiwgYXJ0aWZpY2lhbCBtaXhpbmcgYW5kIGZsdXNoaW5nLCBvbW5pdm9yb3VzIGZpc2ggcmVtb3ZhbCwgYWxnYWVjaWRlIGFwcGxpY2F0aW9ucyBhbmQgc2VkaW1lbnQgZHJlZGdpbmcsIGhhdmUgYmVlbiB1c2VkIHRvIHJlZHVjZSBibG9vbSBvY2N1cnJlbmNlcy4gSG93ZXZlciwgbWFuYWdlcnMgbm93IGZhY2UgdGhlIGFkZGl0aW9uYWwgY2hhbGxlbmdlIG9mIGhhdmluZyB0byBhZGRyZXNzIHRoZSBlZmZlY3RzIG9mIGNsaW1hdGUgY2hhbmdlIG9uIHdhdGVyc2hlZCBoeWRyb2xvZ2ljYWwgYW5kIG51dHJpZW50IGxvYWQgZHluYW1pY3MsIHdhdGVyIHRlbXBlcmF0dXJlLCBtaXhpbmcgcmVnaW1lIGFuZCBpbnRlcm5hbCBudXRyaWVudCBjeWNsaW5nLiBSaXNpbmcgdGVtcGVyYXR1cmVzIGFuZCBpbmNyZWFzaW5nIGZyZXF1ZW5jaWVzIGFuZCBtYWduaXR1ZGVzIG9mIGV4dHJlbWUgd2VhdGhlciBldmVudHMsIGluY2x1ZGluZyB0cm9waWNhbCBjeWNsb25lcywgZXh0cmF0cm9waWNhbCBzdG9ybXMsIGZsb29kcyBhbmQgZHJvdWdodHMsIGFsbCBwcm9tb3RlIEN5YW5vSEFCcyBhbmQgYWZmZWN0IHRoZSBlZmZpY2FjeSBvZiBlY29zeXN0ZW0gcmVtZWRpYXRpb24gbWVhc3VyZXMuIFRoZXNlIGNsaW1hdGljIGNoYW5nZXMgd2lsbCBsaWtlbHkgcmVxdWlyZSBzZXR0aW5nIHN0cmljdGVyIG51dHJpZW50IChpbmNsdWRpbmcgYm90aCBuaXRyb2dlbiBhbmQgcGhvc3Bob3J1cykgcmVkdWN0aW9uIHRhcmdldHMgZm9yIGJsb29tIGNvbnRyb2wgaW4gYWZmZWN0ZWQgd2F0ZXJzLiBJbiBhZGRpdGlvbiwgdGhlIGVmZmljYWN5IG9mIGN1cnJlbnRseSB1c2VkIG1ldGhvZHMgdG8gcmVkdWNlIEN5YW5vSEFCcyB3aWxsIG5lZWQgdG8gYmUgcmUtZXZhbHVhdGVkIGluIGxpZ2h0IG9mIHRoZSBzeW5lcmdpc3RpYyBlZmZlY3RzIG9mIGNsaW1hdGUgY2hhbmdlIHdpdGggbnV0cmllbnQgZW5yaWNobWVudC4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTA3MS9tZjE4MzkyIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2Nkci5saWIudW5jLmVkdS9kb3dubG9hZHMvMDU3NDIyODM0IiwiaXNzdWVkRGF0ZSI6IjIwMTktMDMtMjUiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTEwIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJNYXJpbmUgYW5kIEZyZXNod2F0ZXIgUmVzZWFyY2giLCJpc3N1ZSI6NSwidm9sdW1lIjo3MSwiaW1wYWN0RmFjdG9yIjoiMi43MCJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjU3OSwibGFzdFBhZ2UiOjU3OX19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJDU0lSTyBQdWJsaXNoaW5nIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjoxMDQsImNpdGVkQnlDb3VudCI6OTF9fSwicXVvdGUiOiJUaGF0IHJlbGF0aW9uc2hpcCBoYXMgc2VydmVkIGFzIHRoZSBmb3VuZGF0aW9uIGZvciBzZXR0aW5nICBudXRyaWVudCBsb2FkaW5nIHRhcmdldHMgaW4gbGFrZXMgYXJvdW5kIHRoZSB3b3JsZCwgYW5kIGNhcmVmdWxseSAgZGVzaWduZWQgZXhwZXJpbWVudHMgbWF5IGVsdWNpZGF0ZSB0byB3aGF0IGV4dGVudCB0ZW1wZXJhdHVyZSAgaW5jcmVhc2UgYWZmZWN0cyB0aGUgdGhyZXNob2xkIGF0IHdoaWNoIHRoZSBmcmVxdWVuY3kgb2Ygb2NjdXIgIHJlbmNlIG9mIEN5YW5vSEFCcyBxdWlja2x5IHJpc2VzIHdpdGggaW5jcmVhc2luZyBOIGFuZCBQLiBDb25jbHVzaW9ucyBSZWNlbnQgc3R1ZGllcyBpbmRpY2F0ZSB0aGF0IHRoZSBzeW5lcmdpc3RpYyBjb21iaW5hdGlvbiBvZiBhbnRocm9wb2dlbmljIG51dHJpZW50IGxvYWRpbmcsIHJpc2luZyB0ZW1wZXJhdHVyZXMsIGVuaGFuY2VkICB2ZXJ0aWNhbCBzdHJhdGlmaWNhdGlvbiwgaW5jcmVhc2VkIHJlc2lkZW5jZSB0aW1lIGFuZCBtb3JlICBleHRyZW1lIGNsaW1hdGljIGNvbmRpdGlvbnMgb3ZlcmFsbCB3aWxsIGZhdm91ciBjeWFub2JhY3RlcmlhbCAgZG9taW5hbmNlIGluIGEgd2lkZSByYW5nZSBvZiBhcXVhdGljIGVjb3N5c3RlbXMuIFRoZSBleHBhbiAgc2lvbiBvZiBDeWFub0hBQnMgaGFzIHNlcmlvdXMgY29uc2VxdWVuY2VzIGZvciBodW1hbiB3YXRlciAgc3VwcGxpZXMsIGZpc2hlcmllcyBhbmQgcmVjcmVhdGlvbmFsIHJlc291cmNlcy4gRWNvc3lzdGVtIHNjYWxlICBwaHlzaWNhbCBjaGVtaWNhbCBiaW90aWMgbWl0aWdhdGlvbiBzdHJhdGVnaWVzIHdpbGwgaGF2ZSB0byAgaW5jb3Jwb3JhdGUgbnV0cmllbnQgKG1vc3Qgb2Z0ZW4gYm90aCBOIGFuZCBQKSBpbnB1dCByZWR1Y3Rpb25zLiAgVGhlIG1hZ25pdHVkZXMgb2YgdGhlc2UgcmVkdWN0aW9ucyB3aWxsIG5lZWQgdG8gYmUgc3lzdGVtICBzcGVjaWZpYyBhbmQgd2lsbCBsaWtlbHkgbmVlZCB0byBiZSBhZGp1c3RlZCB3aXRoIGNoYW5naW5nIGNsaSAgbWF0aWMgY29uZGl0aW9ucy4gU3BlY2lmaWNhbGx5LCB3YXRlciBxdWFsaXR5IG1hbmFnZXJzIHdpbGwgaGF2ZSAgdG8gYWNjb21tb2RhdGUgd2FybWluZyBhbmQgZ3JlYXRlciBoeWRyb2xvZ2ljYWwgdmFyaWFiaWxpdHkgIGFuZCBleHRyZW1lbmVzcyByZXN1bHRpbmcgZnJvbSBjbGltYXRlIGNoYW5nZSBpbiBmb3JtdWxhdGluZyAgdGhlaXIgc3RyYXRlZ2llcy4gQSBrZXkgbG9uZyB0ZXJtIGNvbnRyb2wgd2UgY2FuIGV4ZXJ0IHRvIHJlZHVjZSAgdGhlIHJhdGUgYW5kIGV4dGVudCBvZiBnbG9iYWwgd2FybWluZyBpcyBjdXJiaW5nIGdyZWVuaG91c2UgZ2FzICBlbWlzc2lvbnMuIFdpdGhvdXQgdGhpcyBlc3NlbnRpYWwgc3RlcCwgZnV0dXJlIHdhcm1pbmcgdHJlbmRzLCAgaHlkcm9sb2dpY2FsIGV4dHJlbWVzIChyZWNvcmQgcmFpbmZhbGxzIGFuZCBkcm91Z2h0cykgYW5kIHRoZWlyICBlZmZlY3RzIG9uIGFxdWF0aWMgZWNvc3lzdGVtcyB3aWxsIGNvbnRpbnVlIHRvIHByb21vdGUgb3Bwb3IgIHR1bmlzdGljIEN5YW5vSEFCcy4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6MTQuMTQ5MjMzODE4MDU0MiwibGVmdCI6MTMuMTI4NTk1MzUyMTcyODUyLCJ3aWR0aCI6MzcuMDgyMTcyMzkzNzk4ODMsImhlaWdodCI6MS41MTY2MDIyNzc3NTU3MzczfX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjE1LjQ1MTUwODUyMjAzMzY5MSwibGVmdCI6MTMuMTI4NTQ0ODA3NDM0MDgyLCJ3aWR0aCI6MzcuMDg0ODMxMjM3NzkyOTcsImhlaWdodCI6MS41MTY2MDMyMzE0MzAwNTM3fX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjE2Ljc1Mzc4NDE3OTY4NzUsImxlZnQiOjEzLjEyODU0MjkwMDA4NTQ1LCJ3aWR0aCI6MzcuMDgzMjA5OTkxNDU1MDgsImhlaWdodCI6MS41MTY2MDQxODUxMDQzNzAxfX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjE4LjA1NjA1MzE2MTYyMTA5NCwibGVmdCI6MTMuMTI4NTQ0ODA3NDM0MDgyLCJ3aWR0aCI6MzYuNjE1MDc0MTU3NzE0ODQ0LCJoZWlnaHQiOjEuNTE2NjA0MTg1MTA0MzcwMX19LHsicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjoxOS40NjgzNzgwNjcwMTY2LCJsZWZ0IjoxMy4xMjg1NDQ4MDc0MzQwODIsIndpZHRoIjozNC4yOTA1MzExNTg0NDcyNjYsImhlaWdodCI6MS4yNDUwMTEyMTA0NDE1ODk0fX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjIyLjIyNjE0Mjg4MzMwMDc4LCJsZWZ0IjoxMy4xMjg1NTA1Mjk0Nzk5OCwid2lkdGgiOjcuNjQ5ODczNzMzNTIwNTA4LCJoZWlnaHQiOjEuMTM1NDgzMTQ1NzEzODA2Mn19LHsicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjoyMy43MzQ4NDk5Mjk4MDk1NywibGVmdCI6MTMuMTI4NTM1MjcwNjkwOTE4LCJ3aWR0aCI6MzcuMDg5Mjg2ODA0MTk5MjIsImhlaWdodCI6MjYuMDg4NjQwMjEzMDEyNjk1fX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNTg5In19XQ%3D%3D. Reviving TEK aligns with national initiatives like Amrit Sarovars, aiming to restore 75,000 water bodies by 2024, blending ancestral insights with contemporary monitoring to preempt blooms.

Indian landscapes abound with TEK exemplars tailored to lake ecosystems. In the Himalayan foothills, sacred lakes like Rewalsar were maintained through ritualistic cleanups and riparian afforestation, curbing phosphorus recycling under warming conditions (Kumar & Mahajan, 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlBhd2FuIiwiZmFtaWx5IjoiS3VtYXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW1icmlzaCBLdW1hciIsImZhbWlseSI6Ik1haGFqYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyNTIzLTM5NjMiLCIyNTIzLTM5NzEiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMDcvczQyNDUyLTAyMC0zMDgyLTgiLCJtYWciOiIzMDM1OTYwMjI4Iiwib3BlbmFsZXgiOiJXMzAzNTk2MDIyOCJ9LCJ0aXRsZSI6IlRyb3BoaWMgc3RhdHVzIGFuZCBpdHMgcmVndWxhdGluZyBmYWN0b3IgZGV0ZXJtaW5hdGlvbiBhdCB0aGUgUmV3YWxzYXIgTGFrZSwgbm9ydGh3ZXN0IEhpbWFsYXlhIChIUCksIEluZGlhLCBiYXNlZCBvbiBzZWxlY3RlZCBwYXJhbWV0ZXJzIGFuZCBtdWx0aXZhcmlhdGUgc3RhdGlzdGljYWwgYW5hbHlzaXMiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAwNy9zNDI0NTItMDIwLTMwODItOCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9saW5rLnNwcmluZ2VyLmNvbS9jb250ZW50L3BkZi8xMC4xMDA3L3M0MjQ1Mi0wMjAtMzA4Mi04LnBkZiIsImlzc3VlZERhdGUiOiIyMDIwLTA2LTIyIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiU04gQXBwbGllZCBTY2llbmNlcyIsImlzc3VlIjo3LCJ2b2x1bWUiOjIsImltcGFjdEZhY3RvciI6IjcuMzUifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiU3ByaW5nZXIgTmF0dXJlIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo0MCwiY2l0ZWRCeUNvdW50IjoxOX19LCJxdW90ZSI6IkNoYW5naW5nIHF1YWxpdHkgaGFzIGNyZWF0ZWQgdGhlIHdhdGVyIHNjYXJjaXR5IGFuZCAgYSBodWdlIG1hc3Mgb2YgcG9wdWxhdGlvbiB0aHVzIGJlZW4gZGVwcml2ZWQgZnJvbSB1c2luZyAgdGhlIGZyZXNod2F0ZXIgZm9yIHZhcmlvdXMgcHVycG9zZXMgWzMsIDRdLiBIb3dldmVyLCB0aGUgIG9ibm94aW91cyBtYXRlcmlhbHMgdGhhdCBzdGF5IGluIHdhdGVyIGJvZGllcyBmb3IgYSBsb25nLXRpbWUgcGVyaW9kIGJyaW5nIGNoYW5nZXMgaW4gdGhlIHBoeXNpb2NoZW1pY2FsIGNoYXItYWN0ZXJpc3RpY3MgYXMgd2VsbCBhcyB0aGUgbmF0dXJhbCBzY2VuYXJpbyBvZiB0aGUgbGFrZXMgWzVdLiAgVGhlIEhpbWFsYXlhbiBsYWtlcyBhcmUgdW5kZXIgc3RyZXNzIGR1ZSB0byBtaXNtYW5hZ2VkIGh1bWFuIGFjdGl2aXRpZXMsIHdoaWNoIGluY3JlYXNlIG51dHJpZW50cyBhbmQgcG9sbHV0aW9uIGluIHRlcm1zIG9mIGhlYXZ5IG1ldGFscyB0b3hpY2l0eSBhbmQgYmFjdGVyaWFsIGNvbnRhbWluYXRpb24gIFs24oCTOF0uIFRoZSBtYW5hZ2VtZW50IGFuZCBwcmVzZXJ2YXRpb24gb2YgdGhlIGxha2VzIGF0ICBwcmVzZW50IGhhdmUgYmVjb21lIGEgZ3JlYXQgY2hhbGxlbmdlIGZvciBlbnZpcm9ubWVudGFsLWlzdHMgYW5kIG1hbmFnZXJzIFs1LCA5XS4gVGhlIGV1dHJvcGhpY2F0aW9uIGlzIGFsc28gYSBtYWluIHByb2JsZW0gb2YgdGhlICBmcmVzaHdhdGVyIGxha2VzLCBhbmQgaXQgaXMgc3BlY2lmaWVkIGZvciB0aGUgbGFrZXMgaGF2aW5nICBudXRyaWVudCBlbnJpY2htZW50LiBJdCBjYXVzZXMgZXhjZXNzaXZlIGdyb3d0aCBvZiBhbGdhZSAgYW5kIGlzIGNvbnNpZGVyZWQgdG8gYmUgcmVzcG9uc2libGUgZm9yIGRlZ3JhZGluZyB0aGUgIHF1YWxpdGF0aXZlIG5hdHVyZSBvZiB0aGUgbGFrZXMgWzUsIDEwXS4gVGhlIGV1dHJvcGhpY2F0aW9uICBpcyBmb2xsb3dlZCBieSB0aGUgYWxnYWwgYmxvb20sIHdoaWNoIGdlbmVyYXRlcyB0b3hpbiwgIHdvcnNlbnMgd2F0ZXIgcXVhbGl0eSwgZGVwbGV0ZXMgZGlzc29sdmVkIG94eWdlbiAoRE8pICogUGF3YW4gS3VtYXIsIHBhd2FuLmV2c0BnbWFpbC5jb20gfCAxRGVwYXJ0bWVudCBvZiBFbnZpcm9ubWVudGFsIFNjaWVuY2VzLCBDZW50cmFsIFVuaXZlcnNpdHkgb2YgSGltYWNoYWwgUHJhZGVzaCwgRGhhcmFtc2hhbGEgMTc2MjE1LCBJbmRpYS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2OC41Nzg0NjA2OTMzNTkzOCwibGVmdCI6OC41NzE0MTg3NjIyMDcwMzEsIndpZHRoIjo0MC4zMjc1MTQ2NDg0Mzc1LCJoZWlnaHQiOjEuNTE5ODU1MjYwODQ4OTk5fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NzAuMTU4OTk2NTgyMDMxMjUsImxlZnQiOjguNTY5NzM4Mzg4MDYxNTIzLCJ3aWR0aCI6NDAuMzU0NzI0ODg0MDMzMiwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjcxLjczOTU0NzcyOTQ5MjE5LCJsZWZ0Ijo4LjU2OTczODM4ODA2MTUyMywid2lkdGgiOjQwLjM0OTY5MzI5ODMzOTg0NCwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjczLjMyMDA5MTI0NzU1ODYsImxlZnQiOjguNTY4MDU5OTIxMjY0NjQ4LCJ3aWR0aCI6NDAuMDIwMDg4MTk1ODAwNzgsImhlaWdodCI6MS41MTk4NTUyNjA4NDg5OTl9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo3NC45MDA2NDIzOTUwMTk1MywibGVmdCI6OC41NjgwNTk5MjEyNjQ2NDgsIndpZHRoIjozOS45OTk5Mjc1MjA3NTE5NSwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjc2LjQ4MTE4NTkxMzA4NTk0LCJsZWZ0Ijo4LjU2ODA1NzA2MDI0MTcsIndpZHRoIjo0MC4zNTcwNzQ3Mzc1NDg4MywiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjc4LjA2MTcyOTQzMTE1MjM0LCJsZWZ0Ijo4LjU2OTczNjQ4MDcxMjg5LCJ3aWR0aCI6NDAuMzUzNzEzOTg5MjU3ODEsImhlaWdodCI6MS41MTk4NTUyNjA4NDg5OTl9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2MC42NzU3Mjc4NDQyMzgyOCwibGVmdCI6NTEuNDI3MTY1OTg1MTA3NDIsIndpZHRoIjo0MC4zNzg1NjY3NDE5NDMzNiwiaGVpZ2h0IjoxOC45MDU4NTUxNzg4MzMwMDh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo4MS45MjczMzc2NDY0ODQzOCwibGVmdCI6OC40MDE0OTc4NDA4ODEzNDgsIndpZHRoIjo3My45NTQ2OTY2NTUyNzM0NCwiaGVpZ2h0IjoyLjY3ODc2MjY3NDMzMTY2NX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEifX1d. Village tanks (pokharis) in Bihar and Odisha, such as those around Chilika Lagoon, historically filtered sewage via constructed wetlands and fish polycultures, mitigating salinity shifts and Microcystis dominance from aquaculture (Jilbert et al., 2020; Nair & Nayak, 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hdnlhIFZpa3JhbWFuIiwiZmFtaWx5IjoiTmFpciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQcmF0ZWVwIEt1bWFyIiwiZmFtaWx5IjoiTmF5YWsifX1dLCJpZHMiOnsiaXNzbiI6WyIyMjk2LTc3NDUiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzODkvZm1hcnMuMjAyMy4xMDg3Mjk2Iiwib3BlbmFsZXgiOiJXNDM4MjcyMzEzMyJ9LCJ0aXRsZSI6IlVuY292ZXJpbmcgd2F0ZXIgcXVhbGl0eSBhbmQgZXZhbHVhdGluZyB2dWxuZXJhYmlsaXRpZXMgb2Ygc21hbGwtc2NhbGUgZmlzaGVyaWVzIGluIENoaWxpa2EgTGFnb29uLCBJbmRpYSIsImFic3RyYWN0IjoiU21hbGwtc2NhbGUgZmlzaGVyaWVzIChTU0ZzKSBwbGF5IGEgc3Ryb25nIHJvbGUgaW4gc3VzdGFpbmluZyBtaWxsaW9ucyBvZiBsaXZlbGlob29kcywgZm9vZCBzZWN1cml0eSwgbnV0cml0aW9uLCBhbmQgaW5jb21lIGdsb2JhbGx5IGJ1dCB0aGUgZmlzaGVycyBlbmdhZ2VkIGluIHRoaXMgc2VjdG9yIHNpbXVsdGFuZW91c2x5IGV4cGVyaWVuY2UgaGlnaCBsZXZlbHMgb2YgdnVsbmVyYWJpbGl0eSBhbmQgcHJvY2Vzc2VzIG9mIG1hcmdpbmFsaXNhdGlvbi4gU2V2ZXJhbCBmYWN0b3JzIGFyZSBhdHRyaWJ1dGVkIHRvIHRoZSBtdWx0aWRpbWVuc2lvbmFsIHZ1bG5lcmFiaWxpdGllcyB0aGUgc21hbGwtc2NhbGUgZmlzaGVycyBleHBlcmllbmNlLCBzcGFubmluZyBib3RoIG5hdHVyYWwgKGUuZy4sIG5hdHVyYWwgZGlzYXN0ZXJzLCBlY29zeXN0ZW0gY2hhbmdlKSBhbmQgYW50aHJvcG9nZW5pYyAoZS5nLiwgcG9saWN5IGNoYW5nZSwgaHlkcm9sb2dpY2FsIGludGVydmVudGlvbnMsIGFxdWFjdWx0dXJlKSBwcmVzc3VyZXMuIFdoaWxlIHRoZXJlIGlzIG11Y2ggbGl0ZXJhdHVyZSBvbiB2YXJpb3VzIG5hdHVyYWwgYW5kIGh1bWFuIGRyaXZlcnMgb2YgdnVsbmVyYWJpbGl0eSBpbiBzbWFsbC1zY2FsZSBmaXNoZXJ5IGNvbW11bml0aWVzLCBhbiBhYnNlbmNlIG9mIHJlc2VhcmNoIGNvbm5lY3RpbmcgdnVsbmVyYWJpbGl0eSB3aXRoIHdhdGVyIHF1YWxpdHkgaXMgZXZpZGVudC4gRmlzaGVyIGNvbW11bml0aWVzIG9mdGVuIHRhbGsgYWJvdXQgZmlzaCBpbiByZWxhdGlvbiB0byB0aGUgaGVhbHRoIG9mIHRoZWlyIGFxdWF0aWMgaGFiaXRhdHMgd2hlcmVpbiB3YXRlciBxdWFsaXR5IGlzIHNlZW4gYXMgYSBrZXkgcGFyYW1ldGVyLiBUaGUgbGluayBiZXR3ZWVuIGhlYWx0aHkgZmlzaCBhbmQgZ29vZCBxdWFsaXR5IHdhdGVyIGhhcyBzaWduaWZpY2FudCBpbXBsaWNhdGlvbnMgZm9yIHN0cm9uZyBhbmQgdmlhYmxlIGZpc2hpbmcgY29tbXVuaXRpZXMuIFRoaXMgcGFwZXIgZXhhbWluZXMgdGhlc2UgbGlua3MgZnVydGhlciBieSBmb2N1c2luZyBvbiB0aGUgbmF0dXJlIG9mIHZ1bG5lcmFiaWxpdGllcyBjYXVzZWQgYnkgd2F0ZXIgcXVhbGl0eSBjaGFuZ2VzIGluIHRoZSBzbWFsbC1zY2FsZSBmaXNoZXJ5IHN5c3RlbSBvZiBDaGlsaWthIExhZ29vbiBpbiBJbmRpYS4gV2UgdW5kZXJ0YWtlIGRldGFpbGVkIGFuYWx5c2lzIG9mIHRoZSBpbnZhc2l2ZSBzaHJpbXAgYXF1YWN1bHR1cmUgYWN0aXZpdGllcyBhbmQgaHlkcm9sb2dpY2FsIGludGVydmVudGlvbnMgZm9yIG9wZW5pbmcgb2YgYSBsYWdvb24gaW5sZXQgd2l0aCB0aGUgQmF5IG9mIEJlbmdhbCBhcyB0d28gZG9taW5hbnQgZHJpdmVycyBhZHZlcnNlbHkgaW1wYWN0aW5nIHdhdGVyIHF1YWxpdHkgYW5kIGluY3JlYXNpbmcgdnVsbmVyYWJpbGl0aWVzIG9mIHRoZSBlbnRpcmUgc21hbGwtc2NhbGUgZmlzaGVyaWVzIHNvY2lhbC1lY29sb2dpY2FsIHN5c3RlbS4gT3VyIGFuYWx5c2lzIHN1Z2dlc3RzIHRoYXQgdGhlcmUgYXJlIHN0cm9uZyBpbnRlcmNvbm5lY3Rpb25zIGJldHdlZW4gY2hhbmdlcyBpbiB3YXRlciBxdWFsaXR5IGFuZCB0aGUgbGV2ZWxzIG9mIHZ1bG5lcmFiaWxpdGllcyBpbiB0aGUgU1NGcyBvZiBDaGlsaWthIExhZ29vbi4gUG9sbHV0YW50cyBzdWNoIGFzIHBlc3RpY2lkZXMsIGFuZCBvcmdhbmljIGNvbXBvdW5kcyBhY2N1bXVsYXRlIGluIGZpc2ggdGlzc3VlcyBhbmQgYWZmZWN0IHRoZWlyIGdyb3d0aCwgcmVwcm9kdWN0aW9uLCBhbmQgb3ZlcmFsbCBoZWFsdGguIFRoaXMgbGVkIHRvIGRlY2xpbmVzIGluIGZpc2ggcG9wdWxhdGlvbnMsIG1ha2luZyBpdCBtb3JlIGRpZmZpY3VsdCBmb3IgZmlzaGVycyB0byBtYWtlIGEgbGl2aW5nLiBJbiBhZGRpdGlvbiB0byBkaXJlY3QgaW1wYWN0cyBvbiBmaXNoIHBvcHVsYXRpb25zLCBwb29yIHdhdGVyIHF1YWxpdHkgYWxzbyBoYXMgaW5kaXJlY3QgZWZmZWN0cyBvbiB0aGUgc29jaWFsIGFuZCBlY29ub21pYyB2dWxuZXJhYmlsaXR5IG9mIFNTRnMuIEZvciBleGFtcGxlLCBjb250YW1pbmF0aW9uIG9mIHdhdGVyIHNvdXJjZXMgbGVkIHRvIHRoZSByZWR1Y2VkIG51bWJlciBvZiBmaXNoIHNwZWNpZXMgcmVkdWNpbmcgdGhlIGFtb3VudCBvZiB0aW1lIGZpc2hlcnMgY2FuIHNwZW5kIG9uIGZpc2hpbmcgYWN0aXZpdGllcy4gVGhpcyBhbHNvIGFmZmVjdGVkIHRoZSBtYXJrZXRhYmlsaXR5IG9mIGZpc2ggcHJvZHVjdHMsIHJlZHVjZWQgaW5jb21lIGFuZCBpbmNyZWFzZWQgcG92ZXJ0eS4gVG8gZnVsbHkgdW5kZXJzdGFuZCB0aGUgaW50ZXJjb25uZWN0aW9ucyBiZXR3ZWVuIHdhdGVyIHF1YWxpdHkgYW5kIHZ1bG5lcmFiaWxpdHkgaW4gU1NGcyBpbiBDaGlsaWthIExhZ29vbiwgaXQgaXMgaW1wb3J0YW50IHRvIGNvbnNpZGVyIGJvdGggZW52aXJvbm1lbnRhbCBhbmQgc29jaWFsIGZhY3RvcnMsIGFzIHdlbGwgYXMgdGhlIGNvbXBsZXggZmVlZGJhY2sgbG9vcHMgYmV0d2VlbiB0aGVzZSBmYWN0b3JzLiBUaGUgc3R1ZHkgaGVscHMgaW4gYnJpZGdpbmcgYSBjcnVjaWFsIGdhcCBpbiBvdXIgdW5kZXJzdGFuZGluZyBvZiB0aGUgcm9sZSBvZiB3YXRlciBxdWFsaXR5IGluIHZ1bG5lcmFiaWxpdHkgYW5hbHlzaXMgd2l0aGluIHJlc291cmNlIGRlcGVuZGVudCBjb21tdW5pdGllcy4gV2UgY29uY2x1ZGUgd2l0aCBrZXkgaW5zaWdodHMgb24gcG9zc2libGUgY29waW5nIHJlc3BvbnNlcyBhbmQgYWRhcHRpdmUgY2FwYWNpdHkgbmVjZXNzYXJ5IGZvciB0aGUgc21hbGwtc2NhbGUgZmlzaGVyaWVzIGNvbW11bml0aWVzIHRvIHRyYW5zaXRpb24gdG93YXJkIHZpYWJpbGl0eS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMzM4OS9mbWFycy4yMDIzLjEwODcyOTYiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vd3d3LmZyb250aWVyc2luLm9yZy9hcnRpY2xlcy8xMC4zMzg5L2ZtYXJzLjIwMjMuMTA4NzI5Ni9wZGYiLCJpc3N1ZWREYXRlIjoiMjAyMy0wNi0yOSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZyb250aWVycyBpbiBNYXJpbmUgU2NpZW5jZSIsInZvbHVtZSI6MTAsImltcGFjdEZhY3RvciI6IjQuMjgifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRnJvbnRpZXJzIE1lZGlhIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo1MSwiY2l0ZWRCeUNvdW50Ijo1fX0sInF1b3RlIjoiV2F0ZXIgcXVhbGl0eSBvZiBDaGlsaWthIExhZ29vbiBoYXMgZGVncmFkZWQgZHVlIHRvIGFudGhyb3BvZ2VuaWMgaW50ZXJ2ZW50aW9ucyBhbmQgbmF0dXJhbCBmb3JjZXMgb2YgY2hhbmdlLiBWYXJpb3VzIGFjdGl2aXRpZXMgaW4gdGhlIENoaWxpa2EgTGFnb29uLCBzdWNoIGFzIGluZHVzdHJpYWwgd2FzdGV3YXRlciBkaXNwb3NhbCwgc2V3YWdlIGR1bXBpbmcsIGFxdWFjdWx0dXJlLCBoeWRyb2xvZ2ljYWwgaW50ZXJ2ZW50aW9ucywgYW5kIGN5Y2xvbmVzLCBhcmUgcHJvZHVjaW5nIHNhbGluaXR5IGNoYW5nZXMsIHNlZGltZW50IGRlcG9zaXRpb24sIG51dHJpZW50IGVucmljaG1lbnQsIGV1dHJvcGhpY2F0aW9uLCBhbmQgZGVhZCB6b25lcywgYXMgc3RhdGVkLiBGaXNoIHN1cnZpdmFsIGlzIGF0IHJpc2sgYXMgYSByZXN1bHQgb2YgdGhlIHNpZ25pZmljYW50IHJlZHVjdGlvbiBpbiBlbnZpcm9ubWVudGFsIGNpcmN1bXN0YW5jZXMsIHdoaWNoIGRpbWluaXNoZXMgZmlzaGVybWVu4oCZcyBpbmNvbWUgYW5kIHB1c2hlcyB0aGVtIG91dCBvZiBzb2NpZXR5LiBUaGVzZSBjYXVzZXMgYWZmZWN0aW5nIHRoZSB3YXRlciBxdWFsaXR5IG9mIHRoZSBMYWdvb24gZGVwaWN0IG1hbnkgZmFjZXMgb2YgdnVsbmVyYWJpbGl0eSBpbiBTU0YgY29tbXVuaXRpZXMuIEFzIHRoZSBDaGlsaWthIExhZ29vbiBlbnZpcm9ubWVudCBpcyBleHBlY3RlZCB0byB1bmRlcmdvIGluY3JlYXNpbmdseSBkcmFzdGljIGNoYW5nZXMgKHN1Y2ggYXMgY2F0Y2ggY29tcG9zaXRpb24sIGNhdGNoIGNhcGFjaXR5LCBiaW9kaXZlcnNpdHkgdmFyaWF0aW9ucywgd2F0ZXIgcXVhbGl0eSwgYW5kIGZpc2hlcnkgcmV2ZW51ZXMpLCBpbnRlcm5hdGlvbmFsIHBvbGljeSBoYXMgYmVjb21lIG1vcmUgaW50ZXJlc3RlZCBpbiBhZGFwdGF0aW9uIG1lYXN1cmVzLiBHaXZlbiBob3cgZGl2ZXJzZSBTU0YgZWNvc3lzdGVtcyBhcmUgZXhwZWN0ZWQgdG8gYmUgYmV0d2VlbiBjb250aW5lbnRzLCB0aGVyZSBpcyBubyDigJxvbmUgc2l6ZSBmaXRzIGFsbOKAnSBhcHByb2FjaC4gVGhlIHBhcGVyIG91dGxpbmVkIHNldmVyYWwgY29waW5nIGFuZCBhZGFwdGF0aW9uIG1lY2hhbmlzbXMgdG8gaW1wcm92ZSB3YXRlciBxdWFsaXR5IGFuZCBjb250aW51ZSBmaXNoaW5nIGluIGEgc3VzdGFpbmFibGUgd2F5LiBBZGFwdGF0aW9uIHN0cmF0ZWdpZXMgY2FuIG9jY2FzaW9uYWxseSBoYXZlIHVuZm9yZXNlZW4gY29uc2VxdWVuY2VzIGZvciBmaXNoZXJpZXMgYW5kIGZpc2hpbmcgY29tbXVuaXRpZXMuIEFueSB1bmV4cGVjdGVkIGNvbnNlcXVlbmNlcyBvZiBhZGFwdGF0aW9uIGNhbiBiZSBtaXRpZ2F0ZWQgdGhyb3VnaCBsb25nLXRlcm0gcGxhbm5pbmcgYW5kIGlkZW50aWZpY2F0aW9uIG9mIGZ1dHVyZSBvcHRpb25zIHRocm91Z2ggc2NlbmFyaW8gYW5hbHlzaXMuIFRoZSBmaW5kaW5ncyBvZiB0aGlzIHBhcGVyIGNhbiBiZSB1dGlsaXplZCB0byBkZXZlbG9wIHBvbGljaWVzIHRvIHByb3RlY3Qgd2F0ZXIgcXVhbGl0eSBhbmQgU1NGcyBpbiBhIHZhcmlldHkgb2YgY29hc3RhbCBsYWdvb24gc2V0dGluZ3MuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjIyLjI4NTAzNjA4NzAzNjEzMywibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ5MzA1MzQzNjI3OTMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjIzLjc3MzUxOTUxNTk5MTIxLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDgzMzY0MTA1MjI0NjEsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjI1LjI1NTU4NDcxNjc5Njg3NSwibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ4OTU1OTE3MzU4Mzk4NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjYuNzQ0MDY4MTQ1NzUxOTUzLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNTA2ODA1NDE5OTIxODc1LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyOC4yMjYxMzMzNDY1NTc2MTcsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40OTA0NjMyNTY4MzU5NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjkuNzE0NjE2Nzc1NTEyNjk1LCJsZWZ0Ijo4LjMzMTU5MjU1OTgxNDQ1Mywid2lkdGgiOjQwLjQ5NDA5ODY2MzMzMDA4LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozMS4xOTY2ODM4ODM2NjY5OTIsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40OTQzOTIzOTUwMTk1MywiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MzIuNjg1MTY1NDA1MjczNDQsImxlZnQiOjguMzMxNTk2Mzc0NTExNzE5LCJ3aWR0aCI6NDAuNDg3NDQ1ODMxMjk4ODMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM0LjE2NzIzMjUxMzQyNzczNCwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40OTc3NDU1MTM5MTYwMTYsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM1LjY1NTcxNTk0MjM4MjgxLCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ5ODYzODE1MzA3NjE3LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozNy4xMzc3ODMwNTA1MzcxMSwibGVmdCI6MTAuNjg1MTg2Mzg2MTA4Mzk4LCJ3aWR0aCI6MzguMTQxMTA1NjUxODU1NDcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM4LjYyNjI2NjQ3OTQ5MjE5LCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4NzE1MjA5OTYwOTM3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDAuMTA4MzMzNTg3NjQ2NDg0LCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4OTg1NjcxOTk3MDcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQxLjU5NjgxMzIwMTkwNDMsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDg4NjY2NTM0NDIzODMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQzLjA3ODg4MDMxMDA1ODU5NCwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40ODE4NjExMTQ1MDE5NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDQuNTY3MzYzNzM5MDEzNjcsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDgzMDU4OTI5NDQzMzYsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQ2LjA0OTQzMDg0NzE2Nzk3LCJsZWZ0Ijo4LjMzMzEwODkwMTk3NzUzOSwid2lkdGgiOjQwLjUwMDc0NzY4MDY2NDA2LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo0Ny41Mzc5MTQyNzYxMjMwNSwibGVmdCI6OC4zMzMxMDg5MDE5Nzc1MzksIndpZHRoIjo0MC40OTU2MDU0Njg3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDkuMDE5OTgxMzg0Mjc3MzQ0LCJsZWZ0Ijo4LjMzMzEwNjA0MDk1NDU5LCJ3aWR0aCI6NDAuNTA2ODE2ODY0MDEzNjcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUwLjUwODQ2NDgxMzIzMjQyLCJsZWZ0Ijo4LjMzMzEwNjA0MDk1NDU5LCJ3aWR0aCI6NDAuNDg4NTA2MzE3MTM4NjcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUxLjk5MDUyODEwNjY4OTQ1LCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDk3NzI2NDQwNDI5NjksImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUzLjQ3OTAxMTUzNTY0NDUzLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6MzMuMTU5NDQ2NzE2MzA4NTk0LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo1NC45NjEwNzg2NDM3OTg4MywibGVmdCI6MTAuNjgzNjcwMDQzOTQ1MzEyLCJ3aWR0aCI6MzguMTM2NTk2Njc5Njg3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NTYuNDQ5NTYyMDcyNzUzOTA2LCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDg3MTQ0NDcwMjE0ODQ0LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEzIn19LHsib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRvbSIsImZhbWlseSI6IkppbGJlcnQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUmFvdWzigJBNYXJpZSIsImZhbWlseSI6IkNvdXR1cmUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQnJpYW4iLCJmYW1pbHkiOiJIdXNlciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLYWxldmkiLCJmYW1pbHkiOiJTYWxvbmVuIn19XSwiaWRzIjp7Imlzc24iOlsiMDAxOC04MTU4IiwiMTU3My01MTE3Il0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDA3L3MxMDc1MC0wMjAtMDQ0NTcteCIsIm1hZyI6IjMxMDA0NTkwNTAiLCJvcGVuYWxleCI6IlczMTAwNDU5MDUwIn0sInRpdGxlIjoiUHJlZmFjZTogUmVzdG9yYXRpb24gb2YgZXV0cm9waGljIGxha2VzOiBjdXJyZW50IHByYWN0aWNlcyBhbmQgZnV0dXJlIGNoYWxsZW5nZXMiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAwNy9zMTA3NTAtMDIwLTA0NDU3LXgiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vbGluay5zcHJpbmdlci5jb20vY29udGVudC9wZGYvMTAuMTAwNy9zMTA3NTAtMDIwLTA0NDU3LXgucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjAtMTEtMTYiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA3IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJIeWRyb2Jpb2xvZ2lhIiwiaXNzdWUiOjIxLCJ2b2x1bWUiOjg0NywiaW1wYWN0RmFjdG9yIjoiMy4wMCJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjQzNDMsImxhc3RQYWdlIjo0MzU3fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlNwcmluZ2VyIFNjaWVuY2UrQnVzaW5lc3MgTWVkaWEifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjk1LCJjaXRlZEJ5Q291bnQiOjU4fX0sInF1b3RlIjoiU2ltaWxhciBkZWdyYWRhdGlvbiBvZiBmcmVzaHdhdGVyIGVjb3N5c3RlbXMgaXMgb2JzZXJ2ZWQgd29ybGR3aWRlLCB3aXRoIHRoZSB0aW1lbGluZSBvZiBodW1hbiBwcmVzc3VyZSBkaWN0YXRlZCBieSB0cmVuZHMgaW4gdXJiYW5pemF0aW9uIGFuZCBpbnRlbnNpdmUgYWdyaWN1bHR1cmUgaW4gZGlmZmVyZW50IGdlb2dyYXBoaWMgcmVnaW9ucyAoZS5nLiwgVmVubmVtbyBldCBhbC4sIDIwMDkpLiBUaGUgcHJpbmNpcGFsIGRyaXZlciBvZiBlY29zeXN0ZW0gc3RyZXNzIGluIG1vc3QgYXF1YXRpYyBzeXN0ZW1zIGlzIGV1dHJvcGhpY2F0aW9uLCB0aGUgZXhjZXNzIGdyb3d0aCBvZiBhbGdhZSBkcml2ZW4gYnkgYW50aHJvcG9nZW5pYyBpbnB1dHMgb2YgdGhlIG1hY3JvbnV0cmllbnRzIG5pdHJvZ2VuIChOKSBhbmQgcGhvc3Bob3J1cyAoUCkgKFNtaXRoLCAyMDAzOyBCaXJrIGV0IGFsLiwgMjAyMCkuIE51dHJpZW50cyBhcmUgZW1pdHRlZCBmcm9tIGJvdGggcG9pbnQgYW5kIGRpZmZ1c2Ugc291cmNlcyBpbiB0aGUgaHVtYW4gZW52aXJvbm1lbnQgYW5kIGVudGVyIGFxdWF0aWMgc3lzdGVtcyB2aWEgc3VyZmFjZSBydW5vZmYsIGFzIHdlbGwgYXMgZ3JvdW5kLXdhdGVyIGFuZCBhdG1vc3BoZXJpYyBkZXBvc2l0aW9uIChQYWVybCwgMTk5NykuIEV1dHJvcGhpY2F0aW9uIG9mdGVuIGxlYWRzIHRvIHRoZSBkb21pbmFuY2Ugb2YgcHJpbWFyeSBwcm9kdWN0aW9uIGJ5IGN5YW5vYmFjdGVyaWEsIGluY2x1ZGluZyB0b3hpYyBnZW5lcmEgc3VjaCBhcyBEb2xpY2hvc3Blcm11bSwgQXBoYW5pem9tZW5vbiwgYW5kIE1pY3JvY3lzdGlzIChIdWlzbWFuIGV0IGFsLiwgMjAxOCksIGFzIHdlbGwgYXMgb3h5Z2VuIGRlcGxldGlvbiBhbmQgaGFiaXRhdCBkZWdyYWRhdGlvbiBpbiBsYWtlcyAoQXJlbmQgZXQgYWwuLCAyMDExKS4gQmVjYXVzZSBsYWtlcyBwcm92aWRlIGEgcmFuZ2Ugb2YgY3J1Y2lhbCBlY29zeXN0ZW0gc2VydmljZXMgKFdpbHNvbiAmIENhcnBlbnRlciwgMTk5OSksIHRoZXJlIGlzIGEgZ3Jvd2luZyBuZWVkIGZvciBtYW5hZ2VtZW50IGFwcHJvYWNoZXMgdGhhdCByZXN0b3JlIHdhdGVyIHF1YWxpdHkgaW4gZnJlc2h3YXRlciBzeXN0ZW1zLiBXaXRoaW4gdGhlIGZyYW1ld29yayBvZiBVbml0ZWQgTmF0aW9ucyBTdXN0YWluYWJsZSBEZXZlbG9wLW1lbnQgR29hbCA2IChDbGVhbiBXYXRlciBhbmQgU2FuaXRhdGlvbiksIHNldmVyYWwgZGlzdGluY3QgdGFyZ2V0cyByZWxhdGUgdG8gdGhlIGltcHJvdmVtZW50IG9mIHdhdGVyIHF1YWxpdHkgYW5kIHByb3RlY3Rpb24gb2YgYXF1YXRpYyBlY29zeXN0ZW1zIGJ5IDIwMzAgKFVuaXRlZCBOYXRpb25zLCAyMDE1KS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyOC4yOTg4NjQzNjQ2MjQwMjMsImxlZnQiOjguNTQ5MjA1NzgwMDI5Mjk3LCJ3aWR0aCI6MzkuMzcxNDk0MjkzMjEyODksImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MjkuOTkxMTM0NjQzNTU0Njg4LCJsZWZ0Ijo4LjU0OTIwNTc4MDAyOTI5Nywid2lkdGgiOjM5LjM2NTExOTkzNDA4MjAzLCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMxLjY4MzQwNjgyOTgzMzk4NCwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNjk2NjMyMzg1MjUzOSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozMy4zNjc5NzMzMjc2MzY3MiwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNzU2NzkwMTYxMTMyOCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNS4wNjAyNDkzMjg2MTMyOCwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNzA5NjQwNTAyOTI5NywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNi43NTI1MTc3MDAxOTUzMSwibGVmdCI6OC41NDkyMDg2NDEwNTIyNDYsIndpZHRoIjozOS4zNzY5NjA3NTQzOTQ1MywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC40MzcwODgwMTI2OTUzMSwibGVmdCI6OC41NDkyMDg2NDEwNTIyNDYsIndpZHRoIjozOS4zNjY1Njk1MTkwNDI5NywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0MC4xMjkzNTYzODQyNzczNDQsImxlZnQiOjguNTQ5MjA4NjQxMDUyMjQ2LCJ3aWR0aCI6MzkuMzY0Mzc5ODgyODEyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0MS44MjE2MjQ3NTU4NTkzNzUsImxlZnQiOjguNTQ5MjA4NjQxMDUyMjQ2LCJ3aWR0aCI6MzkuMzY1NjczMDY1MTg1NTUsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDMuNTA2MTk1MDY4MzU5Mzc1LCJsZWZ0Ijo4LjU0OTM4OTgzOTE3MjM2Mywid2lkdGgiOjM5LjM3MDM5MTg0NTcwMzEyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0NS4xOTg0NjM0Mzk5NDE0MDYsImxlZnQiOjguNTQ5NTcyOTQ0NjQxMTEzLCJ3aWR0aCI6MzkuMzY1ODQ0NzI2NTYyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ni44OTA3MzU2MjYyMjA3LCJsZWZ0Ijo4LjU0OTU3Mjk0NDY0MTExMywid2lkdGgiOjM5LjM3Mjk1NTMyMjI2NTYyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0OC41NzUzMDIxMjQwMjM0NCwibGVmdCI6OC41NDk1NzI5NDQ2NDExMTMsIndpZHRoIjozOS4zNzI0MjEyNjQ2NDg0NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1MC4yNjc1NzgxMjUsImxlZnQiOjguNTQ5NTc2NzU5MzM4Mzc5LCJ3aWR0aCI6MzkuMzcxMTI0MjY3NTc4MTI1LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjUxLjk1OTg0MjY4MTg4NDc2NiwibGVmdCI6OC41NDk1NzY3NTkzMzgzNzksIndpZHRoIjozOS4zNjc4MzU5OTg1MzUxNTYsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTMuNTc0MTE5NTY3ODcxMDk0LCJsZWZ0Ijo4LjU0OTU3Njc1OTMzODM3OSwid2lkdGgiOjM5LjM3MDc3NzEzMDEyNjk1LCJoZWlnaHQiOjEuMzM2ODk1NTg1MDYwMTE5Nn19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjU1LjI2NjM5MTc1NDE1MDM5LCJsZWZ0Ijo4LjU0OTc2MDgxODQ4MTQ0NSwid2lkdGgiOjM5LjM2NjU4MDk2MzEzNDc2NiwiaGVpZ2h0IjoxLjMzNjg5NTU4NTA2MDExOTZ9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1Ny4wMjg5NTM1NTIyNDYwOTQsImxlZnQiOjguNTQ5OTQ0ODc3NjI0NTEyLCJ3aWR0aCI6MzkuMzYyMDI2MjE0NTk5NjEsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTguNzEzNTIzODY0NzQ2MDk0LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjkuMjk5OTgyMDcwOTIyODUyLCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjYwLjQwNTc5MjIzNjMyODEyNSwibGVmdCI6MTAuNjIyNDU1NTk2OTIzODI4LCJ3aWR0aCI6MzcuMjg3ODY4NDk5NzU1ODYsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NjIuMDk4MDYwNjA3OTEwMTU2LCJsZWZ0Ijo4LjU0OTk0MzkyMzk1MDE5NSwid2lkdGgiOjM5LjM3MzMyNTM0NzkwMDM5LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjYzLjc4MjYzMDkyMDQxMDE1NiwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzMxMjY5ODM2NDI1OCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2NS40NzQ5MDY5MjEzODY3MiwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNjUxMTk5MzQwODIwMywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2Ny4xNjcxNzUyOTI5Njg3NSwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNjk0ODAxMzMwNTY2NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2OC43ODE0NDgzNjQyNTc4MSwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzczMzg0MDk0MjM4MywiaGVpZ2h0IjoxLjMzNjg5MTUzMTk0NDI3NX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjcwLjU0NDAxMzk3NzA1MDc4LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjM5LjM2OTMwODQ3MTY3OTY5LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjcyLjIzNjI4OTk3ODAyNzM0LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjM5LjM3MjAzMjE2NTUyNzM0NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo3My45MjA4NDUwMzE3MzgyOCwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzEzMDczNzMwNDY4NzUsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNDM0NCJ9fV0%3D. Stepwells (baolis) in Gujarat exemplified cascade filtration, trapping sediments and nutrients before they reached downstream lakes like Bellandur, where surfactant foams now trigger HABs (Gobler et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkNocmlzdG9waGVyIEouIiwiZmFtaWx5IjoiR29ibGVyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlJ1dGggVy4iLCJmYW1pbHkiOiJEcmlua3dhdGVyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkEuIFcuIiwiZmFtaWx5IjoiQW50aG9ueSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJKZW5uaWZlciBBLiIsImZhbWlseSI6IkdvbGVza2kifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW5uIE1hcmllIEUuIiwiZmFtaWx5IjoiRmFtdWxhcm8tUGVjb3JhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1pY2hhZWwgQi4iLCJmYW1pbHkiOiJXYWxsYWNlIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5vcmEgUi4gVy4iLCJmYW1pbHkiOiJTdHJhcXVhZGluZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJSb25vam95IiwiZmFtaWx5IjoiSGVtIn19XSwiaWRzIjp7Imlzc24iOlsiMTY2NC0zMDJYIl0sImlzYm4iOltdLCJkb2kiOiIxMC4zMzg5L2ZtaWNiLjIwMjQuMTQ2NDY4NiIsInBtaWQiOiIzOTU2ODk5NyIsIm9wZW5hbGV4IjoiVzQ0MDQxMjA5NTIiLCJleGEiOiJodHRwczovL3d3dy5mcm9udGllcnNpbi5vcmcvam91cm5hbHMvbWljcm9iaW9sb2d5L2FydGljbGVzLzEwLjMzODkvZm1pY2IuMjAyNC4xNDY0Njg2L2Z1bGwifSwidGl0bGUiOiJTZXdhZ2UtYW5kIGZlcnRpbGl6ZXItZGVyaXZlZCBudXRyaWVudHMgYWx0ZXIgdGhlIGludGVuc2l0eSwgZGl2ZXJzaXR5LCBhbmQgdG94aWNpdHkgb2YgaGFybWZ1bCBjeWFub2JhY3RlcmlhbCBibG9vbXMgaW4gZXV0cm9waGljIGxha2VzIiwiYWJzdHJhY3QiOiJDeWFub2JhY3RlcmlhbCBoYXJtZnVsIGFsZ2FsIGJsb29tcyAoQ0hBQnMpIGFyZSBwcm9tb3RlZCBieSBleGNlc3NpdmUgbnV0cmllbnQgbG9hZGluZyBhbmQsIHdoaWxlIGZlcnRpbGl6ZXJzIGFuZCBzZXdhZ2UgYXJlIHRoZSBtb3N0IHByZXZhbGVudCBleHRlcm5hbCBudXRyaWVudCBzb3VyY2VzIGluIG1vc3Qgd2F0ZXJzaGVkcywgdGhlIGRpZmZlcmVudGlhbCBlZmZlY3RzIG9mIHRoZXNlIG51dHJpZW50IHNvdXJjZXMgb24gQ0hBQnMgYXJlIHVua25vd24uIEhlcmUsIHdlIHRyYWNrZWQgQ0hBQnMgYW5kIHBlcmZvcm1lZCBleHBlcmltZW50cyBpbiBmaXZlIGRpc3RpbmN0IGxha2VzIGFjcm9zcyB0aGUgTm9ydGhlcm4gVVMgaW5jbHVkaW5nIExha2UgRXJpZS4gRmVydGlsaXplcnMgd2l0aCBhbW1vbml1bSBhbmQgb3J0aG9waG9zcGhhdGUsIG1lbWJyYW5lICgwLjIgzrxtKS1maWx0ZXJlZCBzZXdhZ2UgKGRvbWluYXRlZCBieSByZWR1Y2VkIGZvcm1zIG9mIG5pdHJvZ2VuKSBzYW5kLWFuZCBtZW1icmFuZS1maWx0ZXJlZCBzZXdhZ2UgKGRvbWluYXRlZCBieSBuaXRyYXRlKSwgYW5kIGFuIGlub3JnYW5pYyBudXRyaWVudCBzb2x1dGlvbiBvZiBhbW1vbml1bSBhbmQgb3J0aG9waG9zcGhhdGUgd2VyZSB1c2VkIGFzIGV4cGVyaW1lbnRhbCBudXRyaWVudCBzb3VyY2VzIGZvciBDSEFCcyBhdCBOLWVxdWl2YWxlbnQsIGVudmlyb25tZW50YWxseSByZWFsaXN0aWMgY29uY2VudHJhdGlvbnMuIFBoeXRvcGxhbmt0b24gY29tbXVuaXRpZXMgd2VyZSBldmFsdWF0ZWQgZmx1b3JvbWV0cmljYWxseSwgbWljcm9zY29waWNhbGx5LCBhbmQgdmlhIGhpZ2ggdGhyb3VnaHB1dCBzZXF1ZW5jaW5nIG9mIHRoZSAxNlMgclJOQSBnZW5lLCBhbmQgbGV2ZWxzIG9mIG1pY3JvY3lzdGluIGFuZCB0aGUgzrQgMTUgTiBjb250ZW50IG9mIHBhcnRpY3VsYXRlIG9yZ2FuaWMgbml0cm9nZW4gKM60UE8gMTUgTikgd2VyZSBxdWFudGlmaWVkLiBGZXJ0aWxpemVyIGFuZCBib3RoIHNvdXJjZXMgb2Ygd2FzdGV3YXRlciBpbmNyZWFzZWQgdGhlIGFidW5kYW5jZSBvZiBjeWFub2JhY3RlcmlhIGluIGFsbCBleHBlcmltZW50cyBhY3Jvc3MgYWxsIGZpdmUgbGFrZXMgKCBwICZhbXA7bHQ7IDAuMDUgZm9yIGFsbCkgd2hlcmVhcyBlZmZlY3RzIG9uIGV1a2FyeW90aWMgcGh5dG9wbGFua3RvbiB3ZXJlIGxpbWl0ZWQuIFNhbmQtZmlsdGVyZWQgc2V3YWdlIGNvbnRhaW5lZCBsZXNzIFAsIG9yZ2FuaWMgbWF0dGVyLCBhbmQgYW1tb25pdW0gYnV0IG1vcmUgbml0cmF0ZSBhbmQgaGFkIGEgMjUlIGxlc3MgcG90ZW50IHN0aW11bGF0b3J5IGVmZmVjdCBvbiBjeWFub2JhY3RlcmlhIHRoYW4gbWVtYnJhbmUtZmlsdGVyZWQgc2V3YWdlLCBzdWdnZXN0aW5nIG5pdHJpZmljYXRpb24gbWF5IHBsYXkgYSByb2xlIGluIHJlZHVjaW5nIENIQUJzLiBGZXJ0aWxpemVyIGluY3JlYXNlZCBtaWNyb2N5c3RpbiBsZXZlbHMgYW5kIGRlY3JlYXNlZCB0aGUgzrRQTyAxNSBOIHdoZXJlYXMgd2FzdGV3YXRlciBpbmNyZWFzZWQgzrRQTyAxNSBOICggcCAmYW1wO2x0OyAwLjA1IGZvciBhbGwpLiBNaWNyb2N5c3RpcyB3YXMgdGhlIGdlbnVzIG1vc3QgY29uc2lzdGVudGx5IHByb21vdGVkIGJ5IG51dHJpZW50IHNvdXJjZXMgKCBwICZhbXA7bHQ7IDAuMDUgaW4gYWxsIGV4cGVyaW1lbnRzKSwgZm9sbG93ZWQgYnkgQ3lhbm9iaXVtICggcCAmYW1wO2x0OyAwLjA1IGluIDUwJSBvZiBleHBlcmltZW50cyksIHdpdGggaW5jcmVhc2VzIGluIE1pY3JvY3lzdGlzIGJpb21hc3MgY29uc2lzdGVudGx5IGVsaWNpdGVkIGJ5IG1lbWJyYW5lLWZpbHRlcmVkIHdhc3Rld2F0ZXIuIENvbGxlY3RpdmVseSwgcmVzdWx0cyBkZW1vbnN0cmF0ZSB0aGF0IGRpZmZlcmluZyB0eXBlcyBvZiBzZXdhZ2UgZGlzY2hhcmdlIGFuZCBmZXJ0aWxpemVycyBjYW4gcHJvbW90ZSBDSEFCIGludGVuc2l0eSBhbmQgdG94aWNpdHksIHdoaWxlIGNvbmN1cnJlbnRseSBhbHRlcmluZyBDSEFCIGRpdmVyc2l0eSBhbmQgzrRQTyAxNSBOLiBXaGlsZSBtZW1icmFuZS1maWx0ZXJlZCBzZXdhZ2UgY29uc2lzdGVudGx5IGZhdm9yZWQgTWljcm9jeXN0aXMgLCB0aGUgZGlzY2hhcmdlIG9mIHNld2FnZSB0aHJvdWdoIHNhbmRzIG11dGVkIGJsb29tIGludGVuc2l0eSBzdWdnZXN0aW5nIHNhbmQtYmVkcyBtYXkgcmVwcmVzZW50IGEgdG9vbCB0byByZW1vdmUga2V5IG51dHJpZW50cyBhbmQgcGFydGlhbGx5IG1pdGlnYXRlIENIQUJzLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzg5L2ZtaWNiLjIwMjQuMTQ2NDY4NiIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cuZnJvbnRpZXJzaW4ub3JnL2pvdXJuYWxzL21pY3JvYmlvbG9neS9hcnRpY2xlcy8xMC4zMzg5L2ZtaWNiLjIwMjQuMTQ2NDY4Ni9wZGYiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMS0wNiIsImFjY2Vzc2VkRGF0ZSI6IjIwMjYtMDIiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZyb250aWVycyBpbiBNaWNyb2Jpb2xvZ3kiLCJ2b2x1bWUiOjE1LCJpbXBhY3RGYWN0b3IiOiIzLjk5In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTQ2NDY4NiwibGFzdFBhZ2UiOjE0NjQ2ODZ9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRnJvbnRpZXJzIE1lZGlhIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo3MywiY2l0ZWRCeUNvdW50Ijo5fX0sInF1b3RlIjoiZGVyaXZlZCBudXRyaWVudHMgYWx0ZXIgdGhlIGludGVuc2l0eSwgZGl2ZXJzaXR5LCBhbmQgdG94aWNpdHkgb2YgaGFybWZ1bCBjeWFub2JhY3RlcmlhbCBibG9vbXMgLi4uIEN5YW5vYmFjdGVyaWFsIGhhcm1mdWwgYWxnYWwgYmxvb21zIChDSEFCcykgYXJlIHByb21vdGVkIGJ5IGV4Y2Vzc2l2ZSBudXRyaWVudCBsb2FkaW5nIGFuZCwgd2hpbGUgZmVydGlsaXplcnMgYW5kIHNld2FnZSBhcmUgdGhlIG1vc3QgcHJldmFsZW50IGV4dGVybmFsIG51dHJpZW50IHNvdXJjZXMgaW4gbW9zdCB3YXRlcnNoZWRzLCB0aGUgZGlmZmVyZW50aWFsIGVmZmVjdHMgb2YgdGhlc2UgbnV0cmllbnQgLi4uIG9yZ2FuaWMgbml0cm9nZW4gKM60UE8xNU4pIHdlcmUgcXVhbnRpZmllZC4gRmVydGlsaXplciBhbmQgYm90aCBzb3VyY2VzIG9mIHdhc3Rld2F0ZXIgaW5jcmVhc2VkIHRoZSBhYnVuZGFuY2Ugb2YgLi4uIHBoeXRvcGxhbmt0b24gd2VyZSBsaW1pdGVkLiBTYW5kLWZpbHRlcmVkIHNld2FnZSBjb250YWluZWQgbGVzcyBQLCBvcmdhbmljIG1hdHRlciwgYW5kIGFtbW9uaXVtIGJ1dCBtb3JlIG5pdHJhdGUgYW5kIC4uLiBkaWZmZXJpbmcgdHlwZXMgb2Ygc2V3YWdlIGRpc2NoYXJnZSBhbmQgZmVydGlsaXplcnMgY2FuIHByb21vdGUgQ0hBQiBpbnRlbnNpdHkgYW5kIHRveGljaXR5LCB3aGlsZSBjb25jdXJyZW50bHkgLi4uIHNld2FnZSB0aHJvdWdoIHNhbmRzIG11dGVkIGJsb29tIGludGVuc2l0eSBzdWdnZXN0aW5nIHNhbmQtYmVkcyBtYXkgcmVwcmVzZW50IGEgdG9vbCB0byByZW1vdmUga2V5IG51dHJpZW50cyBhbmQgLi4uICggV2F0c29uIGV0IGFsLiwgMjAxNiA7IEJ1bGxlcmphaG4gZXQgYWwuLCAyMDE2IDsgSHVpc21hbiBldCBhbC4sIDIwMTggKSBhbmQgZXhjZXNzaXZlIG51dHJpZW50IGxvYWRpbmcgaXMgdGhlIHByb2Nlc3MgbW9zdCBjb21tb25seSBjaXRlZCBhcyB0aGUgZW52aXJvbm1lbnRhbCBmYWN0b3IgcHJvbW90aW5nIENIQUJzICgifV0%3D. These systems leveraged indigenous hydrology observing monsoon rhythms, soil microbiomes, and faunal indicators to sustain fertility without chemical inputs, fostering resilience against interactive stressors like corruption-plagued dredging in Dal Lake (Ji et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkRhb2JpbiIsImZhbWlseSI6IkppIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllhbnhpbmciLCJmYW1pbHkiOiJIYW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTGlhbmdob25nIiwiZmFtaWx5IjoiTG9uZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJYaWFva2FuZyIsImZhbWlseSI6IlhpbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJIdWl5YW4iLCJmYW1pbHkiOiJYdSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTaXFpYW4iLCJmYW1pbHkiOiJRaXUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSmlhbmdodWFpIiwiZmFtaWx5IjoiTWVuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJYaW5neGluZyIsImZhbWlseSI6IlpoYW8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiWWFuYW4iLCJmYW1pbHkiOiJIdWFuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJEZWZ1IiwiZmFtaWx5IjoiTGl1In19XSwiaWRzIjp7Imlzc24iOlsiMjI5Ni03MDFYIl0sImlzYm4iOltdLCJkb2kiOiIxMC4zMzg5L2Zldm8uMjAyMy4xMjk3MDQ3Iiwib3BlbmFsZXgiOiJXNDM5MDYxMDY2OCJ9LCJ0aXRsZSI6Ikh5cG94aWEgYW5kIGl0cyBmZWVkYmFjayByZXNwb25zZSB0byBhbGdhbCBibG9vbXMgYW5kIENINCBlbWlzc2lvbnMgaW4gc3VidHJvcGljYWwgcmVzZXJ2b2lycyIsImFic3RyYWN0IjoiQW50aHJvcG9nZW5pYyBudXRyaWVudCBpbnB1dCBzdGltdWxhdGVzIHN1cmZhY2UgZXV0cm9waGljYXRpb24gYW5kIGFsZ2FsIGJsb29tcyBhbmQgdGh1cyBleGFjZXJiYXRlcyBveHlnZW4gZGVwbGV0aW9uIGFuZCBoeXBveGlhIGluIGFxdWF0aWMgZWNvc3lzdGVtcy4gSG93ZXZlciwgZXhpc3Rpbmcgc3R1ZGllcyBoYXZlIHBhaWQgbGl0dGxlIGF0dGVudGlvbiBvbiB0aGUgdW5kZXJzdGFuZGluZyBvZiB0aGUgZmVlZGJhY2sgcmVsYXRpb25zaGlwcyBhbW9uZyBveHlnZW4gY29uc3VtcHRpb24sIGFsZ2FsIHN1Y2Nlc3Npb24gYW5kIGdyZWVuaG91c2UgZ2FzIGVmZmVjdCBhZnRlciByaXZlciBkYW1taW5nLCBwYXJ0aWN1bGFybHkgaW4gVGhyZWUgR29yZ2VzIFJlc2Vydm9pci4gVGhpcyBmaWVsZCB3b3JrIGludmVzdGlnYXRlZCB0aGUgdGhlcm1hbCBzdHJhdGlmaWNhdGlvbiwgd2F0ZXIgcXVhbGl0eSBhbmQgcGh5dG9wbGFua3RvbiBpbiBmb3VyIHRyaWJ1dGFyaWVzIChQZW5neGkgQmF5IChQWEIpLCBEYW5pbmcgQmF5IChETkIpLCBTaGVubm9uZyBCYXkgKFNOQiksIGFuZCBYaWFuZ3hpIEJheSAoWFhCKSkgb2YgdGhlIFRocmVlIEdvcmdlcyBSZXNlcnZvaXIgKFRHUikgaW4gc3ByaW5nIChBcHJpbCAyNS0yOCwgMjAyMiksIGFuZCBleHBsb3JlZCB0aGUgZHJpdmVycyBvZiBoeXBveGlhIGFuZCBpdHMgZmVlZGJhY2sgcmVzcG9uc2UgdG8gYWxnYWwgYmxvb21zIGFuZCBtZXRoYW5lIChDSCA0ICkgZW1pc3Npb25zLiBUaGUgcmVzdWx0cyBzaG93ZWQgdGhhdCBkdXJpbmcgdGhlIG9ic2VydmF0aW9uIHBlcmlvZCwgRE8sIHRoZXJtYWwgc3RyYXRpZmljYXRpb24sIGFuZCBhbGdhbCBibG9vbXMgd2VyZSBtb3JlIHNldmVyZSBpbiBQWEIuIFRoZSBoaWdoIENoZW1pY2FsIE94eWdlbiBEZW1hbmQgKENPRCBNbiApICgxMS4yNyBtZ%2BKAokwgLTEgKSBhbmQgRGlzc29sdmVkIE9yZ2FuaWMgQ2FyYm9uIChET0MpIGNvbnRlbnQgKDcuNTYgbWfigKJMIC0xICkgaW4gUFhCIHdlcmUgdGhlIG1haW4gY2F1c2VzIG9mIHRoZSBwcm9ncmVzc2l2ZSBoeXBveGlhLiBGdXJ0aGVybW9yZSwgc3Ryb25nZXIgZGVuc2l0eSBzdHJhdGlmaWNhdGlvbiBpbiBQWEIgaW5oaWJpdGVkIHRoZSB2ZXJ0aWNhbCBzdXBwbHkgb2YgRE8gYW5kIGFjY2VsZXJhdGVkIGJvdHRvbSBoeXBveGlhLiBCdXQgdGhlIGRvd25zbG9wZSBkZW5zaXR5IGN1cnJlbnQgcmVwbGVuaXNoZWQgYm90dG9tIERPIGluIHRoZSB1cHBlciByZWFjaGVzIG9mIHRoZSBETkIsIFNOQiwgYW5kIFhYQi4gVGhlIENINCBjb25jZW50cmF0aW9uIHdhcyBnZW5lcmFsbHkgaGlnaGVyIGluIGh5cG94aWEgem9uZSwgZXNwZWNpYWxseSBpbiB3aGVyZSBjeWFub2JhY3RlcmlhIGFyZSB0aGUgZG9taW5hbnQgc3BlY2llcyAoZWcuLCBQWDAzIGFuZCBQWDA0KS4gRGFtIGNvbnN0cnVjdGlvbiBsZWQgdG8gdGhlIHN1Y2Nlc3Npb24gb2YgYWxnYWwtZG9taW5hbnQgc3BlY2llcywgZXhlcnRpbmcgYSBzaWduaWZpY2FudCBpbmZsdWVuY2Ugb24gdGhlIHJpdmVyIGNhcmJvbiBjeWNsZS4gVGhlIGV4cGFuc2lvbiBvZiBjeWFub2JhY3RlcmlhbCBibG9vbXMgYW5kIHRoZSBjb25zZXF1ZW5jZSBvZiBtZXRhYm9saWMgc3Vic3RyYXRlcyBpbmNyZWFzZSBhcmUgZ3JhZHVhbGx5IGluY3JlYXNpbmcgb3h5Z2VuIGRlcHJpdmF0aW9uLCBldmVuIGh5cG94aWEsIGFuZCBlbmhhbmNpbmcgQ0ggNCBwcm9kdWN0aW9uIGluIHJpdmVycy4gVGhlc2UgZmluZGluZ3Mgc3VnZ2VzdCB0aGF0IHRoZSBzdWNjZXNzaW9uIG9mIGFsZ2FsLWRvbWluYW50IHNwZWNpZXMgY2F1c2VkIGJ5IHJpdmVyIGRhbW1pbmcgaGFzIGEgcHJvbm91bmNlZCBwb3NpdGl2ZSBmZWVkYmFjayBlZmZlY3Qgb24gcmVzZXJ2b2lyIGh5cG94aWEgcHJvY2Vzc2VzIGFuZCBncmVlbmhvdXNlIGVmZmVjdC4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMzM4OS9mZXZvLjIwMjMuMTI5NzA0NyIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cuZnJvbnRpZXJzaW4ub3JnL2FydGljbGVzLzEwLjMzODkvZmV2by4yMDIzLjEyOTcwNDcvcGRmP2lzUHVibGlzaGVkVjI9RmFsc2UiLCJpc3N1ZWREYXRlIjoiMjAyNC0wMS0wNSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZyb250aWVycyBpbiBFY29sb2d5IGFuZCBFdm9sdXRpb24iLCJ2b2x1bWUiOjExLCJpbXBhY3RGYWN0b3IiOiI0Ljc4In0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkZyb250aWVycyBNZWRpYSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NDAsImNpdGVkQnlDb3VudCI6NX19LCJxdW90ZSI6Im9nZW5pYyBudXRyaWVudCBpbnB1dCBzdGltdWxhdGVzIHN1cmZhY2UgZXV0cm9waGljYXRpb24gYW5kIGFsZ2FsIGJsb29tcyBhbmQgdGh1cyBleGFjZXJiYXRlcyBveHlnZW4gZGVwbGV0aW9uIGFuZCBoeXBveGlhIGluIGFxdWF0aWMgZWNvc3lzdGVtcy4gSG93ZXZlciwgZXhpc3Rpbmcgc3R1ZGllcyBoYXZlIHBhaWQgbGl0dGxlIGF0dGVudGlvbiBvbiB0aGUgdW5kZXJzdGFuZGluZyBvZiB0aGUgZmVlZGJhY2sgcmVsYXRpb25zaGlwcyBhbW9uZyBveHlnZW4gY29uc3VtcHRpb24sIGFsZ2FsIHN1Y2Nlc3Npb24gYW5kIGdyZWVuaG91c2UgZ2FzIGVmZmVjdCBhZnRlciByaXZlciBkYW1taW5nLCAuLi4gRE8gcGxheXMgYSBrZXkgcm9sZSBpbiB0aGUgYmlvZ2VvY2hlbWljYWwgY3ljbGUgb2Ygc3VyZmFjZSB3YXRlcnMgYW5kIGlzIGEgc2Vuc2l0aXZlIGluZGljYXRvciBvZiBwaHlzaWNhbCBhbmQgYmlvZ2VvY2hlbWljYWwgY2hhbmdlcyBpbiByZXNlcnZvaXIgYXF1YXRpYyBlY29zeXN0ZW1zICggV2V0emVsLCAyMDAxICkuIFdoZW4gd2F0ZXJzIGVudGVyIGFuIGFuYWVyb2JpYyBzdGF0ZSwgYXF1YXRpYyBvcmdhbmlzbXMgZGllIGluIGxhcmdlIG51bWJlcnMgYW5kIHRoZSByZWxlYXNlIG9mIHNlZGltZW50LXJlZHVjaW5nIHBvbGx1dGFudHMgKGUuZy4sIGFtbW9uaWEgbml0cm9nZW4sIC4uLiAyMDIyXSApLCBmdXJ0aGVyIGRldGVyaW9yYXRpbmcgd2F0ZXIgcXVhbGl0eS4gV2l0aCB0aGUgaW5jcmVhc2Ugb2YgcG9sbHV0aW9uIGxvYWRzIGluIHdhdGVyc2hlZHMgYW5kIHRoZSBpbnRlbnNpZmljYXRpb24gb2Ygd2F0ZXIgZXV0cm9waGljYXRpb24sIHdhdGVyIGh5cG94aWEgaGFzIGJlY29tZSBhIHNldmVyZSBnbG9iYWwgZWNvbG9naWNhbCBhbmQgZW52aXJvbm1lbnRhbCBwcm9ibGVtIGluIHJlc2Vydm9pcnMgKCBTdW4gZXTCoGFsLiwgMjAyMSA7IEphaXN3YWwgZXTCoGFsLiwgMjAyMyApLiBGb2N1c2luZyBvbiB0aGUgZHluYW1pYyBjaGFuZ2VzIGluIn1d. Documented in ancient texts like the Arthashastra, such practices prioritized watershed-scale balance, averting the hypoxia that precedes cyanobacterial outbreaks (Jansen et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkpvYWNoaW0iLCJmYW1pbHkiOiJKYW5zZW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiR2F2aW4gTC4iLCJmYW1pbHkiOiJTaW1wc29uIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikdlc2EgQS4iLCJmYW1pbHkiOiJXZXloZW5tZXllciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJMYXVyYSBILiIsImZhbWlseSI6IkjDpHJrw7ZuZW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW5kcmV3IE0uIiwiZmFtaWx5IjoiUGF0ZXJzb24ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUGF1bCBBLiBkZWwiLCJmYW1pbHkiOiJHaW9yZ2lvIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ill2ZXMgVC4iLCJmYW1pbHkiOiJQcmFpcmllIn19XSwiaWRzIjp7Imlzc24iOlsiMTc1OC02NzhYIiwiMTc1OC02Nzk4Il0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDM4L3M0MTU1OC0wMjQtMDIwNTgtMyIsIm9wZW5hbGV4IjoiVzQ0MDAyMDgyMDAiLCJleGEiOiJodHRwczovL3d3dy5uYXR1cmUuY29tL2FydGljbGVzL3M0MTU1OC0wMjQtMDIwNTgtMz9lcnJvcj1jb29raWVzX25vdF9zdXBwb3J0ZWQmY29kZT1hNzRhMjVmZi1hNThkLTRkZjgtOTM2Zi0yNjJlMmUzZTkxOTgifSwidGl0bGUiOiJDbGltYXRlLWRyaXZlbiBkZW94eWdlbmF0aW9uIG9mIG5vcnRoZXJuIGxha2VzIiwiYWJzdHJhY3QiOiJBYnN0cmFjdCBPeHlnZW4gZGVwbGV0aW9uIGNvbnN0aXR1dGVzIGEgbWFqb3IgdGhyZWF0IHRvIGxha2UgZWNvc3lzdGVtcyBhbmQgdGhlIHNlcnZpY2VzIHRoZXkgcHJvdmlkZS4gTW9zdCBvZiB0aGUgd29ybGTigJlzIGxha2VzIGFyZSBsb2NhdGVkICZndDs0NcKwIE4sIHdoZXJlIGFjY2VsZXJhdGVkIGNsaW1hdGUgd2FybWluZyBhbmQgZWxldmF0ZWQgY2FyYm9uIGxvYWRzIG1pZ2h0IHNldmVyZWx5IGluY3JlYXNlIHRoZSByaXNrIG9mIGh5cG94aWEsIGJ1dCB0aGlzIGhhcyBub3QgYmVlbiBzeXN0ZW1hdGljYWxseSBleGFtaW5lZC4gSGVyZSBhbmFseXNpcyBvZiAyLjYgbWlsbGlvbiB3YXRlciBxdWFsaXR5IG9ic2VydmF0aW9ucyBmcm9tIDgsMjg4IGxha2VzIHNob3dzIHRoYXQgYmV0d2VlbiAxOTYwIGFuZCAyMDIyLCBtb3N0IG5vcnRoZXJuIGxha2VzIGV4cGVyaWVuY2VkIHJhcGlkIGRlb3h5Z2VuYXRpb24gc3Ryb25nbHkgbGlua2VkIHRvIGNsaW1hdGUtZHJpdmVuIHByb2xvbmdhdGlvbiBvZiBzdW1tZXIgc3RyYXRpZmljYXRpb24uIE94eWdlbiBsZXZlbHMgZGV0ZXJpb3JhdGVkIG1vc3QgaW4gc21hbGwgbGFrZXMgKCZsdDsxMCBoYSkgb3dpbmcgdG8gdGhlaXIgZ3JlYXRlciB2b2x1bWV0cmljIG94eWdlbiBkZW1hbmQgYW5kIHN1cmZhY2Ugd2FybWluZyByYXRlcywgd2hpbGUgdGhlIGxhcmdlc3QgbGFrZXMgZ2FpbmVkIG94eWdlbiB1bmRlciBtaW5pbWFsIHN0cmF0aWZpY2F0aW9uIGNoYW5nZXMgYW5kIGltcHJvdmVkIGFlcmF0aW9uIGF0IHNwcmluZyBvdmVydHVybnMuIFNlYXNvbmFsIG94eWdlbiBjb25zdW1wdGlvbiByYXRlcyBkZWNsaW5lZCwgZGVzcGl0ZSB3aWRlc3ByZWFkIGJyb3duaW5nLiBQcm9saWZlcmF0aW5nIGFub3hpYSBlbmhhbmNlZCBzZWFzb25hbCBpbnRlcm5hbCBsb2FkaW5nIG9mIEMsIFAgYW5kIE4gYnV0IGRlcGxldGVkIFAgbG9uZy10ZXJtLCBpbmRpY2F0aW5nIHRoYXQgZGVveHlnZW5hdGlvbiBjYW4gZXhoYXVzdCByZWRveC1zZW5zaXRpdmUgZnJhY3Rpb25zIG9mIHNlZGltZW50IG51dHJpZW50IHJlc2Vydm9pcnMuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjEwMzgvczQxNTU4LTAyNC0wMjA1OC0zIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAzOC9zNDE1NTgtMDI0LTAyMDU4LTMiLCJpc3N1ZWREYXRlIjoiMjAyNC0wNy0wMSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTEiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6Ik5hdHVyZSBDbGltYXRlIENoYW5nZSIsImlzc3VlIjo4LCJ2b2x1bWUiOjE0LCJpbXBhY3RGYWN0b3IiOiIxNS4wMiJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjgzMiwibGFzdFBhZ2UiOjgzOH19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJOYXR1cmUgUG9ydGZvbGlvIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo4MCwiY2l0ZWRCeUNvdW50IjozN319LCJxdW90ZSI6IkFlcm9iaWMgcmVzcGlyYXRpb24gY2FuIGRlcGxldGUgb3h5Z2VuIGNvbmNlbnRyYXRpb25zIGF0IGRlcHRoIHdoZW4gc2Vhc29uYWwgc3RyYXRpZmljYXRpb24gcHJldmVudHMgYWVyYXRpb24gb2YgdGhlIHdhdGVyIGNvbHVtbls1XShodHRwczovL3d3dy5uYXR1cmUuY29tL2FydGljbGVzL3M0MTU1OC0wMjQtMDIwNTgtMyNyZWYtQ1I1KS4gVGhpcyBkZXRyaW1lbnRhbCBjb25kaXRpb24sIHRlcm1lZCBoeXBveGlhIChET%2BKAiTzigIky4oCJbWfigIls4oiSMSksIGRpbWluaXNoZXMgdGhlIGNvbGQtd2F0ZXIgaGFiaXRhdCBvZiBhcXVhdGljIGxpZmUsIGluY2x1ZGluZyBjb21tZXJjaWFsbHkgaW1wb3J0YW50IGZpc2hbNl0oaHR0cHM6Ly93d3cubmF0dXJlLmNvbS9hcnRpY2xlcy9zNDE1NTgtMDI0LTAyMDU4LTMjcmVmLUNSNikgYW5kIGNhbiBjYXVzZSBtYXNzIG1vcnRhbGl0eSBpZiBwZXJzaXN0ZW50WzddKGh0dHBzOi8vd3d3Lm5hdHVyZS5jb20vYXJ0aWNsZXMvczQxNTU4LTAyNC0wMjA1OC0zI3JlZi1DUjcpLiBBbm94aWEgKERP4oCJPOKAiTAuNeKAiW1n4oCJbOKIkjEpIHByb21vdGVzIHRoZSByZWR1Y3RpdmUgZGlzc29sdXRpb24gb2YgbWluZXJhbC1ib3VuZCBjYXJib24gYW5kIHBob3NwaG9ydXNbOF0oaHR0cHM6Ly93d3cubmF0dXJlLmNvbS9hcnRpY2xlcy9zNDE1NTgtMDI0LTAyMDU4LTMjcmVmLUNSOCksIFs5XShodHRwczovL3d3dy5uYXR1cmUuY29tL2FydGljbGVzL3M0MTU1OC0wMjQtMDIwNTgtMyNyZWYtQ1I5KSB3aGljaCBmdWVscyBoYXJtZnVsIGFsZ2FsIGJsb29tc1sxMF0oaHR0cHM6Ly93d3cubmF0dXJlLmNvbS9hcnRpY2xlcy9zNDE1NTgtMDI0LTAyMDU4LTMjcmVmLUNSMTApIGFuZCBsaW1pdHMgbWljcm9iaWFsIG94aWRhdGlvbiBvZiBtZXRoYW5lLCBhIHBvdGVudCBncmVlbmhvdXNlIGdhc1sxMV0oaHR0cHM6Ly93d3cubmF0dXJlLmNvbS9hcnRpY2xlcy9zNDE1NTgtMDI0LTAyMDU4LTMjcmVmLUNSMTEpLiJ9XQ%3D%3D.

Modern revival hinges on hybridizing TEK with citizen tools, empowering communities as knowledge custodians. In Maharashtra's Ralegan Siddhi village, Anna Hazare's model revived contour bunding and tree pits, slashing erosion and nutrient runoff into local lakes, with verifiable drops in turbidity akin to global volunteer successes (Foulon et al., 2019; Simon et al., 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkRhbmEgRi4iLCJmYW1pbHkiOiJTaW1vbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJHYWJyaWVsIiwiZmFtaWx5IjoiTXVub3oifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUXVvYyBUdWMiLCJmYW1pbHkiOiJEaW5oIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlN1bmcgVm8iLCJmYW1pbHkiOiJEdXkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiS2F0IiwiZmFtaWx5IjoiS2F2YW5hZ2gifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUm9iZXJ0IEouIiwiZmFtaWx5IjoiU21pdGgifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQmFycnkiLCJmYW1pbHkiOiJIdXNrIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlPDqWJhc3RpZW4iLCJmYW1pbHkiOiJTYXV2w6kifX1dLCJpZHMiOnsiaXNzbiI6WyIyMDU3LTQ5OTEiXSwiaXNibiI6W10sImRvaSI6IjEwLjUzMzQvY3N0cC42NTUiLCJvcGVuYWxleCI6Ilc0MzkwNDA1NzM0In0sInRpdGxlIjoiQWRvcHQgYSBMYWtlOiBTdWNjZXNzZnVsbHkgVHJhY2tpbmcgSGFybWZ1bCBDeWFub2JhY3RlcmlhbCBCbG9vbXMgaW4gQ2FuYWRpYW4gU3VyZmFjZSBXYXRlcnMgVGhyb3VnaCBDaXRpemVuIFNjaWVuY2UiLCJhYnN0cmFjdCI6IlRoZSBwcm9saWZlcmF0aW9uIG9mIGhhcm1mdWwgd2F0ZXJib3JuZSBjeWFub2JhY3RlcmlhbCBhbGdhbCBibG9vbXMsIHNvbWUgb2Ygd2hpY2ggY2FuIHByb2R1Y2UgcG90ZW50IHRveGlucywgcG9zZXMgc2V2ZXJlIHJpc2tzIHRvIGVudmlyb25tZW50YWwgYW5kIGh1bWFuIGhlYWx0aC4gQWNhZGVtaWMgYW5kIGdvdmVybm1lbnRhbCBtb25pdG9yaW5nIGVmZm9ydHMgbWF5IGJlIGNvbnN0cmFpbmVkIGJ5IGJ1ZGdldCwgdGltZSwgYW5kIHN0YWZmLCBhbmQgdGh1cyBtaXNzIG90aGVyd2lzZSBzaWduaWZpY2FudCBwb2xsdXRpb24gZXZlbnRzLiBIZXJlLCB3ZSByZXBvcnQgb24gdGhlIGltcGxlbWVudGF0aW9uIG9mIGEgY2l0aXplbiBzY2llbmNlIHByb2plY3QgdG8gdHJhY2sgaGFybWZ1bCBjeWFub2JhY3RlcmlhbCBibG9vbXMgaW4gbGFrZXMgYW5kIHdhdGVyd2F5cyBhY3Jvc3MgQ2FuYWRhLiBUaHJvdWdoIGJvdGggY3Jvd2Rzb3VyY2luZyBhbmQgY3Jvd2RmdW5kaW5nLCB0aGUgQWRvcHQgYSBMYWtlIChBZG9wdCBhIExha2UgMjAyMikgY2FtcGFpZ24gYWltZWQgdG8gZG9jdW1lbnQgdGhlIHBvdGVudGlhbCBwcmVzZW5jZSBvZiBjeWFub2JhY3RlcmlhIGFuZCB0b3hpbnMgd2l0aCB0aGUgYXNzaXN0YW5jZSBvZiBwYXJ0aWNpcGFudHMsIHRodXMgaW1wcm92aW5nIHB1YmxpYyBhd2FyZW5lc3Mgb2YgdGhlIGlzc3VlIG9mIHdhdGVyIHF1YWxpdHkgcHJlc2VydmF0aW9uLiBVc2luZyBzb2NpYWwgbWVkaWEsIHBhcnRpY2lwYW50cyB3ZXJlIGVuY291cmFnZWQgdG8gcGFydGljaXBhdGUgaW4gdGhlIGluaXRpYXRpdmUgYnkgY29sbGVjdGluZyBzYW1wbGVzIGR1cmluZyBhIGJsb29tIGZyb20gYSBuZWFyYnkgcG9uZCBvciBieSBtYWtpbmcgYSBmaW5hbmNpYWwgY29udHJpYnV0aW9uIHRvIHN1cHBvcnQgdGhlIGluaXRpYXRpdmUuIEFkb3B0IGEgTGFrZSBiZW5lZml0dGVkIGZyb20gdGhlIGFuYWx5dGljYWwgcGxhdGZvcm0gb2YgQWxnYWwgQmxvb21zIFRyZWF0bWVudCwgUmlzayBBc3Nlc3NtZW50LCBQcmVkaWN0aW9ucywgYW5kIFByZXZlbnRpb24gKEFUUkFQUCksIGEgcmVzZWFyY2ggcHJvamVjdCBmb2N1c2VkIG9uIHRoZSBwcmVkaWN0aW9uIGFuZCBtYW5hZ2VtZW50IG9mIGhhcm1mdWwgY3lhbm9iYWN0ZXJpYWwgYmxvb21zLiBUaGUgcHJlc2VuY2Ugb2YgY3lhbm90b3hpbnMsIHdoaWNoIGNhbiBjb25maXJtIHdoZXRoZXIgYSBsYWtlIGhhcyBhIHRveGljIGJsb29tLCB3YXMgZGV0ZXJtaW5lZCB0aHJvdWdoIGhpZ2gtcmVzb2x1dGlvbiBtYXNzIHNwZWN0cm9tZXRyeSBhbmFseXNlcy4gVGhpcyBwYXBlciBwcmVzZW50cyBhbiBvdmVydmlldyBvZiB0aGUgaW1wbGVtZW50YXRpb24gb2YgdGhlIEFkb3B0IGEgTGFrZSBpbml0aWF0aXZlLCB0aGUgY2FtcGFpZ27igJlzIHN0YXR1cywgYW5kIHRoZSBsZXNzb25zIGxlYXJuZWQsIGFuZCBpdCBhcmd1ZXMgdGhlIGltcG9ydGFuY2Ugb2YgY29udGludWFsIG1vbml0b3Jpbmcgb2YgY3lhbm9iYWN0ZXJpYWwgYmxvb21zLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC41MzM0L2NzdHAuNjU1IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3N0b3JhZ2UuZ29vZ2xlYXBpcy5jb20vam5sLXVwLWotY3N0cC1maWxlcy9qb3VybmFscy8xL2FydGljbGVzLzY1NS82NThlOWQ3ZDQ2YTBjLnBkZiIsImlzc3VlZERhdGUiOiIyMDIzLTEyLTI5IiwiYWNjZXNzZWREYXRlIjoiMjAyNS0wOCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiQ2l0aXplbiBTY2llbmNlIFRoZW9yeSBhbmQgUHJhY3RpY2UiLCJpc3N1ZSI6MSwidm9sdW1lIjo4LCJpbXBhY3RGYWN0b3IiOiIzLjEyIn0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6NjYsImxhc3RQYWdlIjo2Nn19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJVYmlxdWl0eSBQcmVzcyJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MjcsImNpdGVkQnlDb3VudCI6Mn19LCJxdW90ZSI6IkluIHRoZSBVbml0ZWQgIFN0YXRlcywgYSB2b2x1bnRlZXIgbW9uaXRvcmluZyBwcm9ncmFtIGRldmVsb3BlZCBieSB0aGUgIFdhYmFzaCBSaXZlciBFbmhhbmNlbWVudCBDb3Jwb3JhdGlvbiBoZWxwZWQgdG8gaWRlbnRpZnkgIHdhdGVyLXF1YWxpdHkgcHJpb3JpdGllcyBhY3Jvc3MgdGhlIHdhdGVyc2hlZCAoTXVlbmljaCAgZXQgYWwuIDIwMTYpLiBJbiBDYW5hZGEsIG9yZ2FuaXphdGlvbnMgYXJlIGNvbW1pdHRlZCAgdG8gbW9uaXRvciBhbmQgcHJvdGVjdCB3YXRlciB0aHJvdWdoIGNyb3dkc291cmNlZCAgbW9uaXRvcmluZyBhbmQga25vd2xlZGdlIHRyYW5zZmVyIGluaXRpYXRpdmVzIChHM0UgIDIwMjI7IE90dGF3YSBSaXZlcmtlZXBlciAyMDIzOyBPdXIgTGl2aW5nIFdhdGVycyAyMDIyOyAgV2F0ZXIgUmFuZ2VycyAyMDIyOyBaaGFvLCBMaXUgYW5kIFdlaSAyMDIwKS5UaGVzZSAgZWZmb3J0cyBhcmUgY29tcGxlbWVudGVkIGJ5IG9wZW4gZnJlZSBwbGF0Zm9ybXMgc3VjaCAgYXMgRGF0YVN0cmVhbSwgd2hlcmUgd2F0ZXIgcXVhbGl0eSBkYXRhIGNvbGxlY3RlZCBmcm9tICB2YXJpb3VzIGNvbW11bml0eS1nYXRoZXJlZCBwcm9ncmFtcyBpcyBjdXJhdGVkLCBzdG9yZWQsICBhbmQgc2hhcmVkIHdpdGggdGhlIHB1YmxpYyAoVGhlIEdvcmRvbiBGb3VuZGF0aW9uIDIwMjMpLiBHaXZlbiB0aGUgY3JpdGljYWwgaW1wb3J0YW5jZSBvZiB3YXRlciByZXNvdXJjZXMgYW5kIHRoZWlyIGRpdmVyc2UgdXNlcywgaW5jbHVkaW5nIGRvbWVzdGljIHdlbGxzLCBtdW5pY2lwYWwgZHJpbmtpbmcgd2F0ZXIgcHJvZHVjdGlvbiwgZmlzaGluZywgcmVjcmVhdGlvbmFsIGFjdGl2aXRpZXMsICBhbmQgY3JvcCBpcnJpZ2F0aW9uLCBpdCBpcyBlc3NlbnRpYWwgdG8gcHJvdGVjdCBhbmQgbW9uaXRvciAgdGhlbS4gTGFrZXMsIG9mdGVuIHVzZWQgZm9yIHJlY3JlYXRpb25hbCBhY3Rpdml0aWVzIGFuZCBhcyAgZHJpbmtpbmcgd2F0ZXIgc291cmNlcywgYXJlIHN1c2NlcHRpYmxlIHRvIGJsb29tcyBvZiBhbGdhZSAgYW5kIGN5YW5vYmFjdGVyaWEsIHBhcnRpY3VsYXJseSBpbiB3YXJtLCBzbG93LW1vdmluZywgYW5kICBudXRyaWVudC1yaWNoIHdhdGVycy4gVGhlc2UgYmxvb21zIGNhbiBtYW5pZmVzdCBhcyBmb2FtLCAgc2N1bSwgbWF0cywgb3IgcGFpbnQtbGlrZSBjb2F0aW5ncyBvbiB0aGUgd2F0ZXLigJlzIHN1cmZhY2UgIGFuZCBjYW4gcHJvZHVjZSBjeWFub3RveGlucyB0aGF0IHBvc2UgaGVhbHRoIHJpc2tzIHRvICBodW1hbnMgYW5kIHdpbGRsaWZlIChGaWd1cmUgMSkuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MTIuNDY0MDcyMjI3NDc4MDI3LCJsZWZ0Ijo1MS44NTQ4ODg5MTYwMTU2MjUsIndpZHRoIjozOC43ODY3NTQ2MDgxNTQzLCJoZWlnaHQiOjEuMzU2ODE4NDM3NTc2Mjk0fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MTQuMTA0OTc4NTYxNDAxMzY3LCJsZWZ0Ijo1MS44NTM0MTY0NDI4NzEwOTQsIndpZHRoIjozOC43ODk2OTE5MjUwNDg4MywiaGVpZ2h0IjoxLjM1NjgxODQzNzU3NjI5NH19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjE1Ljc0NTg4Mzk0MTY1MDM5LCJsZWZ0Ijo1MS44NTM0MTY0NDI4NzEwOTQsIndpZHRoIjozOC43ODY3MzkzNDkzNjUyMzQsImhlaWdodCI6MS4zNTY4MTg0Mzc1NzYyOTR9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoxNy4zODY3ODkzMjE4OTk0MTQsImxlZnQiOjUxLjg1MzQxNjQ0Mjg3MTA5NCwid2lkdGgiOjM4Ljc4ODIxNTYzNzIwNzAzLCJoZWlnaHQiOjEuMzU2ODE4NDM3NTc2Mjk0fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MTkuMDI3Njk0NzAyMTQ4NDM4LCJsZWZ0Ijo1MS44NTM0MTY0NDI4NzEwOTQsIndpZHRoIjozOC43ODgyMTU2MzcyMDcwMywiaGVpZ2h0IjoxLjM1NjgxODQzNzU3NjI5NH19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjIwLjY2ODYwMDA4MjM5NzQ2LCJsZWZ0Ijo1MS44NTM0MTY0NDI4NzEwOTQsIndpZHRoIjozOC43ODUyNzQ1MDU2MTUyMzQsImhlaWdodCI6MS4zNTY4MTg0Mzc1NzYyOTR9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyMi4zMDk1MDU0NjI2NDY0ODQsImxlZnQiOjUxLjg1MzQxNjQ0Mjg3MTA5NCwid2lkdGgiOjM4Ljc4ODIzNDcxMDY5MzM2LCJoZWlnaHQiOjEuMzU2ODE4NDM3NTc2Mjk0fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MjMuOTUwNDEwODQyODk1NTA4LCJsZWZ0Ijo1MS44NTM0MTI2MjgxNzM4Mywid2lkdGgiOjM4Ljc4MzgzMjU1MDA0ODgzLCJoZWlnaHQiOjEuMzU2ODE4NDM3NTc2Mjk0fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MjUuNTkxMzE2MjIzMTQ0NTMsImxlZnQiOjUxLjg0OTAwNjY1MjgzMjAzLCJ3aWR0aCI6MzguNzkxMTcyMDI3NTg3ODksImhlaWdodCI6MS4zNTY4MTg0Mzc1NzYyOTR9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyNy4yMzIyMjE2MDMzOTM1NTUsImxlZnQiOjUxLjg1MTk0MDE1NTAyOTMsIndpZHRoIjozOC43OTI2MTc3OTc4NTE1NiwiaGVpZ2h0IjoxLjM1NjgxODQzNzU3NjI5NH19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI4Ljg3MzEyNjk4MzY0MjU3OCwibGVmdCI6NTEuODUxOTQwMTU1MDI5Mywid2lkdGgiOjM4Ljc4Mzc3NTMyOTU4OTg0NCwiaGVpZ2h0IjoxLjM1NjgxODQzNzU3NjI5NH19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMwLjUxNDAzNDI3MTI0MDIzNCwibGVmdCI6NTEuODUxOTQwMTU1MDI5Mywid2lkdGgiOjM4Ljc4ODIzMDg5NTk5NjA5NCwiaGVpZ2h0IjoxLjM1NjgxODQzNzU3NjI5NH19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMyLjE1NDkzNzc0NDE0MDYyNSwibGVmdCI6NTEuODUxOTQwMTU1MDI5Mywid2lkdGgiOjM4LjU4NTI4OTAwMTQ2NDg0NCwiaGVpZ2h0IjoxLjM1NjgxODQzNzU3NjI5NH19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMzLjc5NTg0NTAzMTczODI4LCJsZWZ0Ijo1My43MDQ4Nzk3NjA3NDIxOSwid2lkdGgiOjM2LjkzNTI3OTg0NjE5MTQwNiwiaGVpZ2h0IjoxLjM1NjgxODQzNzU3NjI5NH19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjM1LjQzNjc1MjMxOTMzNTk0LCJsZWZ0Ijo1MS44NTE5NDAxNTUwMjkzLCJ3aWR0aCI6MzguNzg5NzA3MTgzODM3ODksImhlaWdodCI6MS4zNTY4MTg0Mzc1NzYyOTR9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNy4wNzc2NjM0MjE2MzA4NiwibGVmdCI6NTEuODUxOTQwMTU1MDI5Mywid2lkdGgiOjM4Ljc4Njc2NjA1MjI0NjA5NCwiaGVpZ2h0IjoxLjM1NjgxODQzNzU3NjI5NH19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjM4LjcxODU3MDcwOTIyODUxNiwibGVmdCI6NTEuODUxOTQwMTU1MDI5Mywid2lkdGgiOjM4Ljc4MzgxMzQ3NjU2MjUsImhlaWdodCI6MS4zNTY4MTg0Mzc1NzYyOTR9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0MC4zNTk0ODE4MTE1MjM0NCwibGVmdCI6NTEuODUxOTQwMTU1MDI5Mywid2lkdGgiOjM4Ljc4Njc3MzY4MTY0MDYyNSwiaGVpZ2h0IjoxLjM1NjgxODQzNzU3NjI5NH19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQyLjAwMDM5MjkxMzgxODM2LCJsZWZ0Ijo1MS44NTE5NDAxNTUwMjkzLCJ3aWR0aCI6MzguNzg1MzAxMjA4NDk2MDk0LCJoZWlnaHQiOjEuMzU2ODE4NDM3NTc2Mjk0fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDMuNjQxMzAwMjAxNDE2MDE2LCJsZWZ0Ijo1MS44NTE5NDAxNTUwMjkzLCJ3aWR0aCI6MzguNzkxMTUyOTU0MTAxNTYsImhlaWdodCI6MS4zNTY4MTg0Mzc1NzYyOTR9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0NS4yODIyMDc0ODkwMTM2NywibGVmdCI6NTEuODUxOTQwMTU1MDI5Mywid2lkdGgiOjM4Ljc4OTcwNzE4MzgzNzg5LCJoZWlnaHQiOjEuMzU2ODE4NDM3NTc2Mjk0fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDYuOTIzMTE4NTkxMzA4NTk0LCJsZWZ0Ijo1MS44NTE5NDAxNTUwMjkzLCJ3aWR0aCI6MzguNzkyNjI5MjQxOTQzMzYsImhlaWdodCI6MS4zNTY4MTg0Mzc1NzYyOTR9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0OC41NjQwMjk2OTM2MDM1MTYsImxlZnQiOjUxLjg1MTk0MDE1NTAyOTMsIndpZHRoIjozOC43ODUyNzgzMjAzMTI1LCJoZWlnaHQiOjEuMzU2ODE4NDM3NTc2Mjk0fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTAuMjA0OTM2OTgxMjAxMTcsImxlZnQiOjUxLjg1MTk0MDE1NTAyOTMsIndpZHRoIjozOC43ODY3NjYwNTIyNDYwOTQsImhlaWdodCI6MS4zNTY4MTg0Mzc1NzYyOTR9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI2NyJ9fSx7Im9yaWdpbiI6NCwid29yayI6eyJ0eXBlIjo4LCJzdWJ0eXBlIjp7Im9uZW9mS2luZCI6ImFydGljbGVUeXBlIiwiYXJ0aWNsZVR5cGUiOjF9LCJjb250cmlidXRvcnMiOlt7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiLDiXRpZW5uZSIsImZhbWlseSI6IkZvdWxvbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBbGFpbiBOLiIsImZhbWlseSI6IlJvdXNzZWF1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkdsZW5uIiwiZmFtaWx5IjoiQmVub3kifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUmViZWNjYSBMLiIsImZhbWlseSI6Ik5vcnRoIn19XSwiaWRzIjp7Imlzc24iOlsiMTIwMS0zMDgwIiwiMjQwOC05NDQzIl0sImlzYm4iOltdLCJkb2kiOiIxMC4yMTY2L3dxcmouMjAxOS4wMTMiLCJtYWciOiIyOTczODYwMjgxIiwib3BlbmFsZXgiOiJXMjk3Mzg2MDI4MSJ9LCJ0aXRsZSI6IkEgZ2xvYmFsIHNjYW4gb2YgaG93IHRoZSBpc3N1ZSBvZiBudXRyaWVudCBsb2FkaW5nIGFuZCBoYXJtZnVsIGFsZ2FsIGJsb29tcyBpcyBiZWluZyBhZGRyZXNzZWQgYnkgZ292ZXJubWVudHMsIG5vbi1nb3Zlcm5tZW50YWwgb3JnYW5pemF0aW9ucywgYW5kIHZvbHVudGVlcnMiLCJhYnN0cmFjdCI6IkFic3RyYWN0IEhhcm1mdWwgYWxnYWwgYmxvb21zIChIQUJzKSBpbiBhcXVhdGljIGVjb3N5c3RlbXMgYXJlIG9mIGNvbmNlcm4gd29ybGR3aWRlLiBUaGlzIHJldmlldyBkZWFscyB3aXRoIGhvdyBqdXJpc2RpY3Rpb25zIGFyb3VuZCB0aGUgd29ybGQgYXJlIGFkZHJlc3NpbmcgdGhpcyB3YXRlciBxdWFsaXR5IGlzc3VlIHRvIGluZm9ybSByZWNvbW1lbmRhdGlvbnMgcmVnYXJkaW5nIG51dHJpZW50IGxvYWRpbmcgYW5kIEhBQnMgaW4gTWlzc2lzcXVvaSBCYXktTGFrZSBDaGFtcGxhaW4gYW5kIExha2UgTWVtcGhyZW1hZ29nOyB0cmFuc2JvdW5kYXJ5IGxha2VzIGxvY2F0ZWQgaW4gdGhlIFVTQSBhbmQgQ2FuYWRhIHRoYXQgc3VmZmVyIGZyb20gc3ltcHRvbXMgb2YgZXV0cm9waGljYXRpb24uIEEgZ2xvYmFsIHNjYW4gb2YgdGhlIGxpdGVyYXR1cmUgcmVzdWx0ZWQgaW4gdGhlIGNvbnNpZGVyYXRpb24gb2YgMTIgY2FzZSBzdHVkaWVzIG9mIGxhcmdlIHdhdGVyIGJvZGllcyB3aXRoaW4gbGFyZ2Ugd2F0ZXJzaGVkcywgZXhjbHVkaW5nIGluLWxha2UgZ2VvZW5naW5lZXJpbmcgYXBwcm9hY2hlcy4gQWx0aG91Z2ggYWxsIG9mIHRoZSBzeXN0ZW1zIGV4cGVyaWVuY2UgZXhjZXNzaXZlIG51dHJpZW50IGxvYWRpbmcsIHRoZXkgdmFyeSBpbiB0d28ga2V5IHdheXM6IHNvdXJjZXMgb2YgbnV0cmllbnRzIGFuZCBtYW5pZmVzdGF0aW9ucyBvZiBldXRyb3BoaWNhdGlvbiByYW5naW5nIGZyb20gSEFCcywgdG8gbGltaXRlZCByZWNyZWF0aW9uYWwgdXNlcywgdG8gdGhlIGFkZGl0aW9uYWwgY29tcGxleGl0eSBvZiBpbnRlcm5hbCBsb2FkaW5ncyBhbmQgZmlzaCBraWxscywgdXAgdG8gZHJpbmtpbmcgd2F0ZXIgc2h1dGRvd25zLiBUaGUgY2FzZSBzdHVkaWVzIHdlcmUgYW5hbHl6ZWQgd2l0aCByZXNwZWN0IHRvIGZvdXIgY2F0ZWdvcmllcyBvZiBhcHByb2FjaGVzLCBuYW1lbHk6IChpKSByZWd1bGF0b3J5OyAoaWkpIGluY2VudGl2ZS1iYXNlZDsgKGlpaSkgcmlzayBtaXRpZ2F0aW9uOyBhbmQgKGl2KSBvdXRyZWFjaCwgZW5nYWdlbWVudCwgYW5kIGVkdWNhdGlvbmFsLiBXZSBmb3VuZCB0aGF0IHRoZSBtYW5hZ2VtZW50IGZyYW1ld29ya3MgYXJlIGJhc2VkIG9uIGludGVncmF0ZWQgd2F0ZXJzaGVkIG1hbmFnZW1lbnQgcGxhbm5pbmcgYW5kIG5hdGlvbmFsIHN0YW5kYXJkcy4gTmF0aW9uYWwgd2F0ZXIgcXVhbGl0eSBzdGFuZGFyZHMsIGhvd2V2ZXIsIGFyZSBub3Qgc3RyaW5nZW50IGVub3VnaCB0byBwcmV2ZW50IEhBQnMuIE92ZXJhbGwsIGlkZW50aWZpZWQgY2FzZSBzdHVkaWVzIGRpZCBub3Qgc3VjY2Vzc2Z1bGx5IHJlbWVkaWF0ZSBIQUJzLCB0aGV5IHNpbXBseSBtYW5hZ2VkIHRoZW0uIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjIxNjYvd3Fyai4yMDE5LjAxMyIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9pd2Fwb25saW5lLmNvbS93cXJqL2FydGljbGUtcGRmLzU1LzEvMS83MDkyNjEvd3FyamMwNTUwMDAxLnBkZiIsImlzc3VlZERhdGUiOiIyMDE5LTA5LTE3IiwiYWNjZXNzZWREYXRlIjoiMjAyNS0wOSIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiV2F0ZXIgUXVhbGl0eSBSZXNlYXJjaCBKb3VybmFsIiwiaXNzdWUiOjEsInZvbHVtZSI6NTUsImltcGFjdEZhY3RvciI6IjcuODMifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjoxLCJsYXN0UGFnZSI6MjN9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiVVdBIFB1Ymxpc2hpbmcifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjg0LCJjaXRlZEJ5Q291bnQiOjIwfX0sInF1b3RlIjoiV2UgcHJvcG9zZSBzb2x1dGlvbnMgYW5kIHBhdGhzIGZvcndhcmQgdG8gYSB3aWNrZWQgcHJvYmxlbSB3aGljaCBzaG91bGQgcHJvdmUgaW5zaWdodGZ1bCBmb3IgcG9saWN5LW1ha2VycyBhbmQgbWFuLSBhZ2VycyBhbGlrZSB0aHJvdWdoIHRoZSBmb2xsb3dpbmcgZmluZGluZ3MuIE5hdGlvbmFsIHdhdGVyIHF1YWxpdHkgc3RhbmRhcmRzIGhhdmUgbm90IHByb3ZlZCBzdHJpbmdlbnQgZW5vdWdoIHRvIHByZS0gdmVudCBIQUJzLiBJZGVudGlmaWVkIGZyYW1ld29ya3MgYXJlIGFsbCBiYXNlZCBvbiBpbnRlZ3JhdGVkIHdhdGVyc2hlZCBtYW5hZ2VtZW50IGFuZCBuYXRpb25hbCBzdGFuZGFyZHMuIE5vIGFjdHVhbCBzdWNjZXNzIHN0b3JpZXMgcmVsYXRlZCB0byByZW1lZGlhdGlvbiBvZiBIQUJzIHdlcmUgaWRlbnRpZmllZDsgdGhleSB3ZXJlIHNpbXBseSBtYW5hZ2VtZW50IHN0b3JpZXMuIFRoZSBlZmZpY2FjeSBvZiBOUFMgQk1QcyBpcyBxdWVzdGlvbmVkIHRocm91Z2hvdXQgdGhlIHdvcmxkIGR1ZSB0byBpbnN1ZmZpY2llbnQgZXZpZGVuY2UgdG8gZGV0ZXJtaW5lIHdoZXRoZXIgdGhleSBjYW4gbWl0aWdhdGUgSEFCcy4gTWFya2V0LWJhc2VkIHN0cmF0ZWdpZXMgdG8gb2Zmc2V0IHBvbGx1dGlvbiBvciBwcm9tb3RlIHRhcmdldGVkIEJNUCBhZG9wdGlvbiBhcmUgYmVjb21pbmcgcG9wdWxhciBhbmQgcmVzdWx0aW5nIGluIGNvc3QtZWZmZWN0aXZlIGFuZCBlZmZpY2llbnQgYXBwcm9hY2hlcy4gQ29zdOKAk2JlbmVmaXQgYW5hbHlzZXMgbWF5IGluY3JlYXNlIHRoZSB1c2Ugb2YgdGVjaG5pY2FsIHNvbC0gdXRpb25zIG92ZXIgY29tcHJlaGVuc2l2ZSBhcHByb2FjaGVzLiBBdCB0aGlzIHBvaW50LCB0aGVyZSBpcyBub3QgcmVhbGx5IGEgdW5pcXVlIGNhc2Ugc3R1ZHkgdGhhdCBjb3VsZCBiZSBhcHBsaWVkIHRvIExha2UgQ2hhbXBsYWluIGFuZCBMYWtlIE1lbXBocmVtYWdvZy4gU3BlY2lmaWMgZWxlbWVudHMgb2YgdGhlIGFwcHJvYWNoZXMgaWRlbnRpZmllZCBpbiB0aGlzIHJldmlldyBhbmQgdW5pLSBmeWluZyB0aGVtZXMsIGhvd2V2ZXIsIGhvbGQgcHJvbWlzZXMuIFRoaXMgaXMgd2h5IHdlIGFkdm9jYXRlIGZvciBjb25kdWN0aW5nIGEgc3lzdGVtYXRpYyBsaXRlcmF0dXJlIHJldmlldyB3aXRoIHJlZ2FyZCB0byB0aG9zZSB0aGVtZXMgaW4gb3JkZXIgdG8gZGV2ZWxvcCByZWNvbW1lbi0gZGF0aW9ucyB0YWlsb3JlZCB0byB0aGUgTGFrZSBNZW1waHJlbWFnb2cgYW5kIE1pc3Npc3F1b2kgQmF5LUxha2UgQ2hhbXBsYWluIGFyZWFzLiBBdCBsYXN0LCB3ZSB3b3VsZCBsaWtlIHRvIGFkZHJlc3MgdGhlIGVsZXBoYW50LXNpemVkIHF1ZXMtIHRpb24gaW4gdGhlIHJvb20uIFdpbGwgY2xpbWF0ZSBjaGFuZ2UgaW5jcmVhc2UgSEFCcyBmcmVxdWVuY3k%2FIEludGVyYWN0aXZlIGVmZmVjdHMgb2YgZXV0cm9waGljYXRpb24gYW5kIGNsaW1hdGUgY2hhbmdlIG9uIGhhcm1mdWwgY3lhbm9iYWN0ZXJpYWwgYmxvb21zIGFyZSBjb21wbGV4LiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MTUsImJib3giOnsidG9wIjo4NS42MDU1MTQ1MjYzNjcxOSwibGVmdCI6NDkuNzk0ODcyMjgzOTM1NTUsIndpZHRoIjozOS43MTg2NjIyNjE5NjI4OSwiaGVpZ2h0IjoxLjMyODY3MTMzNjE3NDAxMTJ9fSx7InBhZ2VOdW1iZXIiOjE1LCJiYm94Ijp7InRvcCI6ODcuMzk1MzYyODU0MDAzOSwibGVmdCI6NDkuNzk0ODcyMjgzOTM1NTUsIndpZHRoIjozOS43MDI3MDkxOTc5OTgwNSwiaGVpZ2h0IjoxLjMyODY3MTMzNjE3NDAxMTJ9fSx7InBhZ2VOdW1iZXIiOjE1LCJiYm94Ijp7InRvcCI6ODkuMTg1MjExMTgxNjQwNjIsImxlZnQiOjQ5Ljc5NDg3MjI4MzkzNTU1LCJ3aWR0aCI6MzkuNzAyMDk1MDMxNzM4MjgsImhlaWdodCI6MS4zMjg2NzEzMzYxNzQwMTEyfX0seyJwYWdlTnVtYmVyIjoxNSwiYmJveCI6eyJ0b3AiOjkwLjk3NTA1OTUwOTI3NzM0LCJsZWZ0Ijo0OS43OTQ4NzIyODM5MzU1NSwid2lkdGgiOjM5LjcwNDYyMDM2MTMyODEyNSwiaGVpZ2h0IjoxLjMyODY3MTMzNjE3NDAxMTJ9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MTQuMDI1OTkwNDg2MTQ1MDIsImxlZnQiOjEwLjUwMjMyMzE1MDYzNDc2Niwid2lkdGgiOjM5LjcwNTk1NTUwNTM3MTA5NCwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MTUuODE1ODQxNjc0ODA0Njg4LCJsZWZ0IjoxMC41MDIzMjMxNTA2MzQ3NjYsIndpZHRoIjozOS43MDIyNDc2MTk2Mjg5MDYsImhlaWdodCI6MS4zMjg2Njk0Mjg4MjUzNzg0fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjE3LjYwNTY5MTkwOTc5MDA0LCJsZWZ0IjoxMC41MDIxNzYyODQ3OTAwMzksIndpZHRoIjozOS43MDA5MjM5MTk2Nzc3MzQsImhlaWdodCI6MS4zMjg2Njk0Mjg4MjUzNzg0fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjE5LjM5NTU0NDA1MjEyNDAyMywibGVmdCI6MTAuNTAyMTc2Mjg0NzkwMDM5LCJ3aWR0aCI6MzkuNzA4OTAwNDUxNjYwMTU2LCJoZWlnaHQiOjEuMzI4NjY5NDI4ODI1Mzc4NH19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjoyMS4xODUzOTQyODcxMDkzNzUsImxlZnQiOjEwLjUwMjE3NjI4NDc5MDAzOSwid2lkdGgiOjM5LjcxMDA4MzAwNzgxMjUsImhlaWdodCI6MS4zMjg2Njk0Mjg4MjUzNzg0fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjIyLjk3NTI0NDUyMjA5NDcyNywibGVmdCI6MTAuNTAyMTc2Mjg0NzkwMDM5LCJ3aWR0aCI6MzkuNzA0MDI5MDgzMjUxOTUsImhlaWdodCI6MS4zMjg2Njk0Mjg4MjUzNzg0fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjI0Ljc2NTA5NjY2NDQyODcxLCJsZWZ0IjoxMC41MDIxNzYyODQ3OTAwMzksIndpZHRoIjozOS43MDM4ODAzMTAwNTg1OTQsImhlaWdodCI6MS4zMjg2Njk0Mjg4MjUzNzg0fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjI2LjU1NDk0ODgwNjc2MjY5NSwibGVmdCI6MTAuNTAyMDI3NTExNTk2NjgsIndpZHRoIjozOS43MDM4ODAzMTAwNTg1OTQsImhlaWdodCI6MS4zMjg2Njk0Mjg4MjUzNzg0fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjI4LjM0NDc5OTA0MTc0ODA0NywibGVmdCI6MTAuNTAyMDI3NTExNTk2NjgsIndpZHRoIjozOS43MDAwMzg5MDk5MTIxMSwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MzAuMTM0NjQ3MzY5Mzg0NzY2LCJsZWZ0IjoxMC41MDIwMjc1MTE1OTY2OCwid2lkdGgiOjM5LjcwNDYxNjU0NjYzMDg2LCJoZWlnaHQiOjEuMzI4NjY5NDI4ODI1Mzc4NH19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjozMS45MjQ0OTc2MDQzNzAxMTcsImxlZnQiOjEwLjUwMTg3Nzc4NDcyOTAwNCwid2lkdGgiOjM5LjcwNTc4NzY1ODY5MTQwNiwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MzMuNzE0MzUxNjU0MDUyNzM0LCJsZWZ0IjoxMC41MDE3MjkwMTE1MzU2NDUsIndpZHRoIjozOS43MTkyNDU5MTA2NDQ1MywiaGVpZ2h0IjoxLjMyODY2NzQwMjI2NzQ1Nn19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjozNS41MDQyMDM3OTYzODY3MiwibGVmdCI6MTAuNTAxNzI5MDExNTM1NjQ1LCJ3aWR0aCI6MzkuNzAyNzA5MTk3OTk4MDUsImhlaWdodCI6MS4zMjg2Njc0MDIyNjc0NTZ9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MzcuMjk0MDUyMTI0MDIzNDQsImxlZnQiOjEwLjUwMTcyOTAxMTUzNTY0NSwid2lkdGgiOjM5LjcwNTc5OTEwMjc4MzIsImhlaWdodCI6MS4zMjg2Njc0MDIyNjc0NTZ9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MzkuMDgzOTA0MjY2MzU3NDIsImxlZnQiOjEwLjUwMTcyNTE5NjgzODM3OSwid2lkdGgiOjM5LjcwNjUzNTMzOTM1NTQ3LCJoZWlnaHQiOjEuMzI4NjY3NDAyMjY3NDU2fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjQwLjg3Mzc1NjQwODY5MTQwNiwibGVmdCI6MTAuNTAxNzI3MTA0MTg3MDEyLCJ3aWR0aCI6MzkuNzIxMTUzMjU5Mjc3MzQ0LCJoZWlnaHQiOjEuMzI4NjY3NDAyMjY3NDU2fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjQyLjY2MzYwODU1MTAyNTM5LCJsZWZ0IjoxMC41MDE3MjcxMDQxODcwMTIsIndpZHRoIjozOS43MDMxMzY0NDQwOTE4LCJoZWlnaHQiOjEuMzI4NjY3NDAyMjY3NDU2fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjQ0LjQ0NjIwODk1Mzg1NzQyLCJsZWZ0IjoxMC41MDE3MjcxMDQxODcwMTIsIndpZHRoIjozOS43MDA3NzUxNDY0ODQzNzUsImhlaWdodCI6MS4zMjg2Njc0MDIyNjc0NTZ9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6NDYuMjM2MDYxMDk2MTkxNDA2LCJsZWZ0IjoxMC41MDE3MjcxMDQxODcwMTIsIndpZHRoIjozOS43MDc3MjU1MjQ5MDIzNDQsImhlaWdodCI6MS4zMjg2Njc0MDIyNjc0NTZ9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6NDguMDI1OTEzMjM4NTI1MzksImxlZnQiOjEwLjUwMTcyNzEwNDE4NzAxMiwid2lkdGgiOjI1Ljk0MTY2NzU1Njc2MjY5NSwiaGVpZ2h0IjoxLjMyODY2NzQwMjI2NzQ1Nn19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjo0OS44MTU3NjE1NjYxNjIxMSwibGVmdCI6MTMuMTI1MDE2MjEyNDYzMzc5LCJ3aWR0aCI6MzcuMDgxMTgwNTcyNTA5NzY2LCJoZWlnaHQiOjEuMzI4NjY3NDAyMjY3NDU2fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjUxLjYwNTYxMzcwODQ5NjA5NCwibGVmdCI6MTAuNTAxNzI3MTA0MTg3MDEyLCJ3aWR0aCI6MzkuNzA5MjA1NjI3NDQxNDA2LCJoZWlnaHQiOjEuMzI4NjY3NDAyMjY3NDU2fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjUzLjM5NTQ2MjAzNjEzMjgxLCJsZWZ0IjoxMC41MDE3MjcxMDQxODcwMTIsIndpZHRoIjozOS43MTkyMjMwMjI0NjA5NCwiaGVpZ2h0IjoxLjMyODY2NzQwMjI2NzQ1Nn19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjo1NS4xODUzMTAzNjM3Njk1MywibGVmdCI6MTAuNTAxNzI3MTA0MTg3MDEyLCJ3aWR0aCI6MzkuNzEwNjc4MTAwNTg1OTQsImhlaWdodCI6MS4zMjg2Njc0MDIyNjc0NTZ9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIxNiJ9fV0%3D. Smartphone apps could now digitize oral histories, mapping sacred groves that buffer lakes from non-point pollution, while low-cost kits validate traditional bloom sentinels like fish kills or water discoloration (Paerl et al., 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhhbnMgVy4iLCJmYW1pbHkiOiJQYWVybCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLLkUuIiwiZmFtaWx5IjoiSGF2ZW5zIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik4uUy4iLCJmYW1pbHkiOiJIYWxsIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlQuRy4iLCJmYW1pbHkiOiJPdHRlbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNLiIsImZhbWlseSI6IlpodSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJIYWlodWkiLCJmYW1pbHkiOiJYdSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJHLiIsImZhbWlseSI6IlpodSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJCLiIsImZhbWlseSI6IlFpbiJ9fV0sImlkcyI6eyJpc3NuIjpbXSwiaXNibiI6W10sImRvaSI6IjEwLjE3NjE1LzRua2Qtems5MyIsIm9wZW5hbGV4IjoiVzQzMDcwNjI0MjkifSwidGl0bGUiOiJNaXRpZ2F0aW5nIGEgZ2xvYmFsIGV4cGFuc2lvbiBvZiB0b3hpYyBjeWFub2JhY3RlcmlhbCBibG9vbXM6IENvbmZvdW5kaW5nIGVmZmVjdHMgYW5kIGNoYWxsZW5nZXMgcG9zZWQgYnkgY2xpbWF0ZSBjaGFuZ2UiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTc2MTUvNG5rZC16azkzIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2Nkci5saWIudW5jLmVkdS9kb3dubG9hZHMvMDU3NDIyODM0IiwiaXNzdWVkRGF0ZSI6IjIwMjAtMDEtMDEiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTAxIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJDYXJvbGluYSBEaWdpdGFsIFJlcG9zaXRvcnkgKFVuaXZlcnNpdHkgb2YgTm9ydGggQ2Fyb2xpbmEgYXQgQ2hhcGVsIEhpbGwpIn0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlVuaXZlcnNpdHkgb2YgTm9ydGggQ2Fyb2xpbmEgYXQgQ2hhcGVsIEhpbGwifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjAsImNpdGVkQnlDb3VudCI6MH19LCJxdW90ZSI6IlRoYXQgcmVsYXRpb25zaGlwIGhhcyBzZXJ2ZWQgYXMgdGhlIGZvdW5kYXRpb24gZm9yIHNldHRpbmcgIG51dHJpZW50IGxvYWRpbmcgdGFyZ2V0cyBpbiBsYWtlcyBhcm91bmQgdGhlIHdvcmxkLCBhbmQgY2FyZWZ1bGx5ICBkZXNpZ25lZCBleHBlcmltZW50cyBtYXkgZWx1Y2lkYXRlIHRvIHdoYXQgZXh0ZW50IHRlbXBlcmF0dXJlICBpbmNyZWFzZSBhZmZlY3RzIHRoZSB0aHJlc2hvbGQgYXQgd2hpY2ggdGhlIGZyZXF1ZW5jeSBvZiBvY2N1ciAgcmVuY2Ugb2YgQ3lhbm9IQUJzIHF1aWNrbHkgcmlzZXMgd2l0aCBpbmNyZWFzaW5nIE4gYW5kIFAuIENvbmNsdXNpb25zIFJlY2VudCBzdHVkaWVzIGluZGljYXRlIHRoYXQgdGhlIHN5bmVyZ2lzdGljIGNvbWJpbmF0aW9uIG9mIGFudGhyb3BvZ2VuaWMgbnV0cmllbnQgbG9hZGluZywgcmlzaW5nIHRlbXBlcmF0dXJlcywgZW5oYW5jZWQgIHZlcnRpY2FsIHN0cmF0aWZpY2F0aW9uLCBpbmNyZWFzZWQgcmVzaWRlbmNlIHRpbWUgYW5kIG1vcmUgIGV4dHJlbWUgY2xpbWF0aWMgY29uZGl0aW9ucyBvdmVyYWxsIHdpbGwgZmF2b3VyIGN5YW5vYmFjdGVyaWFsICBkb21pbmFuY2UgaW4gYSB3aWRlIHJhbmdlIG9mIGFxdWF0aWMgZWNvc3lzdGVtcy4gVGhlIGV4cGFuICBzaW9uIG9mIEN5YW5vSEFCcyBoYXMgc2VyaW91cyBjb25zZXF1ZW5jZXMgZm9yIGh1bWFuIHdhdGVyICBzdXBwbGllcywgZmlzaGVyaWVzIGFuZCByZWNyZWF0aW9uYWwgcmVzb3VyY2VzLiBFY29zeXN0ZW0gc2NhbGUgIHBoeXNpY2FsIGNoZW1pY2FsIGJpb3RpYyBtaXRpZ2F0aW9uIHN0cmF0ZWdpZXMgd2lsbCBoYXZlIHRvICBpbmNvcnBvcmF0ZSBudXRyaWVudCAobW9zdCBvZnRlbiBib3RoIE4gYW5kIFApIGlucHV0IHJlZHVjdGlvbnMuICBUaGUgbWFnbml0dWRlcyBvZiB0aGVzZSByZWR1Y3Rpb25zIHdpbGwgbmVlZCB0byBiZSBzeXN0ZW0gIHNwZWNpZmljIGFuZCB3aWxsIGxpa2VseSBuZWVkIHRvIGJlIGFkanVzdGVkIHdpdGggY2hhbmdpbmcgY2xpICBtYXRpYyBjb25kaXRpb25zLiBTcGVjaWZpY2FsbHksIHdhdGVyIHF1YWxpdHkgbWFuYWdlcnMgd2lsbCBoYXZlICB0byBhY2NvbW1vZGF0ZSB3YXJtaW5nIGFuZCBncmVhdGVyIGh5ZHJvbG9naWNhbCB2YXJpYWJpbGl0eSAgYW5kIGV4dHJlbWVuZXNzIHJlc3VsdGluZyBmcm9tIGNsaW1hdGUgY2hhbmdlIGluIGZvcm11bGF0aW5nICB0aGVpciBzdHJhdGVnaWVzLiBBIGtleSBsb25nIHRlcm0gY29udHJvbCB3ZSBjYW4gZXhlcnQgdG8gcmVkdWNlICB0aGUgcmF0ZSBhbmQgZXh0ZW50IG9mIGdsb2JhbCB3YXJtaW5nIGlzIGN1cmJpbmcgZ3JlZW5ob3VzZSBnYXMgIGVtaXNzaW9ucy4gV2l0aG91dCB0aGlzIGVzc2VudGlhbCBzdGVwLCBmdXR1cmUgd2FybWluZyB0cmVuZHMsICBoeWRyb2xvZ2ljYWwgZXh0cmVtZXMgKHJlY29yZCByYWluZmFsbHMgYW5kIGRyb3VnaHRzKSBhbmQgdGhlaXIgIGVmZmVjdHMgb24gYXF1YXRpYyBlY29zeXN0ZW1zIHdpbGwgY29udGludWUgdG8gcHJvbW90ZSBvcHBvciAgdHVuaXN0aWMgQ3lhbm9IQUJzLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjoxNC4xNDkyMzM4MTgwNTQyLCJsZWZ0IjoxMy4xMjg1OTUzNTIxNzI4NTIsIndpZHRoIjozNy4wODIxNzIzOTM3OTg4MywiaGVpZ2h0IjoxLjUxNjYwMjI3Nzc1NTczNzN9fSx7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6MTUuNDUxNTA4NTIyMDMzNjkxLCJsZWZ0IjoxMy4xMjg1NDQ4MDc0MzQwODIsIndpZHRoIjozNy4wODQ4MzEyMzc3OTI5NywiaGVpZ2h0IjoxLjUxNjYwMzIzMTQzMDA1Mzd9fSx7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6MTYuNzUzNzg0MTc5Njg3NSwibGVmdCI6MTMuMTI4NTQyOTAwMDg1NDUsIndpZHRoIjozNy4wODMyMDk5OTE0NTUwOCwiaGVpZ2h0IjoxLjUxNjYwNDE4NTEwNDM3MDF9fSx7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6MTguMDU2MDUzMTYxNjIxMDk0LCJsZWZ0IjoxMy4xMjg1NDQ4MDc0MzQwODIsIndpZHRoIjozNi42MTUwNzQxNTc3MTQ4NDQsImhlaWdodCI6MS41MTY2MDQxODUxMDQzNzAxfX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjE5LjQ2ODM3ODA2NzAxNjYsImxlZnQiOjEzLjEyODU0NDgwNzQzNDA4Miwid2lkdGgiOjM0LjI5MDUzMTE1ODQ0NzI2NiwiaGVpZ2h0IjoxLjI0NTAxMTIxMDQ0MTU4OTR9fSx7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6MjIuMjI2MTQyODgzMzAwNzgsImxlZnQiOjEzLjEyODU1MDUyOTQ3OTk4LCJ3aWR0aCI6Ny42NDk4NzM3MzM1MjA1MDgsImhlaWdodCI6MS4xMzU0ODMxNDU3MTM4MDYyfX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjIzLjczNDg0OTkyOTgwOTU3LCJsZWZ0IjoxMy4xMjg1MzUyNzA2OTA5MTgsIndpZHRoIjozNy4wODkyODY4MDQxOTkyMiwiaGVpZ2h0IjoyNi4wODg2NDAyMTMwMTI2OTV9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIxMSJ9fV0%3D. Pilot projects in Loktak Lake integrate Meitei phumdi management floating mats harvested for compost with satellite anomaly detection, exposing regulatory gaps in non-point controls (Nõges et al., 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRpaW5hIiwiZmFtaWx5IjoiTsO1Z2VzIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hc2ltZSIsImZhbWlseSI6IkphbmF0aWFuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlJlZXQiLCJmYW1pbHkiOiJMYXVnYXN0ZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQZWV0ZXIiLCJmYW1pbHkiOiJOw7VnZXMifX1dLCJpZHMiOnsiaXNzbiI6WyIyMzUxLTk4OTQiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMTYvai5nZWNjby4yMDIwLmUwMTM2OSIsIm1hZyI6IjMxMDU0NDYwOTAiLCJvcGVuYWxleCI6IlczMTA1NDQ2MDkwIiwiZXhhIjoiaHR0cHM6Ly93d3cuc2NpZW5jZWRpcmVjdC5jb20vc2NpZW5jZS9hcnRpY2xlL3BpaS9TMjM1MTk4OTQyMDMwOTEwMCJ9LCJ0aXRsZSI6IlBvc3Qtc292aWV0IGNoYW5nZXMgaW4gbml0cm9nZW4gYW5kIHBob3NwaG9ydXMgc3RvaWNoaW9tZXRyeSBpbiB0d28gbGFyZ2Ugbm9uLXN0cmF0aWZpZWQgbGFrZXMgYW5kIHRoZSBpbXBhY3Qgb24gcGh5dG9wbGFua3RvbiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2ouZ2VjY28uMjAyMC5lMDEzNjkiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2ouZ2VjY28uMjAyMC5lMDEzNjkiLCJpc3N1ZWREYXRlIjoiMjAyMC0xMS0yMCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTEiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6Ikdsb2JhbCBFY29sb2d5IGFuZCBDb25zZXJ2YXRpb24iLCJ2b2x1bWUiOjI0LCJpbXBhY3RGYWN0b3IiOiIzLjU5In0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkVsc2V2aWVyIEJWIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo3MSwiY2l0ZWRCeUNvdW50IjoxNH19LCJxdW90ZSI6IlJhbmRvbSBGb3Jlc3QgYW5hbHlzaXMgaW5kaWNhdGVkIGFuIGltcG9ydGFudCByb2xlIG9mIGxpZ2h0IGxpbWl0YXRpb24gaW4gYm90aCBsYWtlcy4gQ29uc3RhbnRseSBsb3dlciBsZXZlbHMgb2YgTiBpbiB0aGUgZGVlcGVyIExha2UgUGVpcHNpIGZhdm91cmVkIE4yLWZpeGluZyBzcGVjaWVzLCB3aGljaCwgYXMgYSBwYXJhZG94LCBiZWNhbWUgUC1saW1pdGVkLiBBcyBjbGltYXRlIHdhcm1pbmcgcmVpbmZvcmNlcyBldXRyb3BoaWNhdGlvbiBwaGVub21lbmEgaW4gbGFrZXMgYnkgaW5jcmVhc2luZyBpbnRlcm5hbCBudXRyaWVudCBsb2FkaW5nIGFuZCBmYXZvdXJpbmcgYmxvb20tZm9ybWluZyBjeWFub2JhY3RlcmlhLCBtb3JlIHN0cmluZ2VudCBtZWFzdXJlcyB3b3VsZCBiZSBuZWVkZWQgdG8gZnVydGhlciBsaW1pdCBudXRyaWVudCBsb2FkcyAoZXNwZWNpYWxseSB0aGF0IG9mIFApIHRvIGxha2VzIHRocm91Z2ggaW1wcm92ZWQgd2FzdGV3YXRlciB0cmVhdG1lbnQgYW5kIGluY3JlYXNlZCBlZmZpY2llbmN5IG9mIGZlcnRpbGlzZXIgYXBwbGljYXRpb24uIExvbmctdGVybSBjaGFuZ2VzIGluIG51dHJpZW50IGxvYWRpbmcgYW5kIHN0b2ljaGlvbWV0cnkgaW4gd2F0ZXIgYm9kaWVzIHJlZmxlY3QgY2hhbmdlcyBpbiBhZ3JpY3VsdHVyYWwgcHJhY3RpY2VzIGFuZCB3YXN0ZXdhdGVyIHRyZWF0bWVudC4ifV0%3D. Economic incentives, mirroring U.S. fisheries offsets, could reward TEK practitioners for verified data, recouping losses like Bengaluru's INR 500-1000 crore (Berdalet et al., 2017)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkVsaXNhIiwiZmFtaWx5IjoiQmVyZGFsZXQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUmFwaGFlbCBNLiIsImZhbWlseSI6Ikt1ZGVsYSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJFZCIsImZhbWlseSI6IlVyYmFuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhlbnJpayIsImZhbWlseSI6IkVuZXZvbGRzZW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTmVpbCBTLiIsImZhbWlseSI6IkJhbmFzIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkVpbGVlbiIsImZhbWlseSI6IkJyZXNuYW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTWljaGVsZSBBLiIsImZhbWlseSI6IkJ1cmZvcmQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiS2VpdGgiLCJmYW1pbHkiOiJEYXZpZHNvbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJDaHJpc3RvcGhlciBKLiIsImZhbWlseSI6IkdvYmxlciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJCZW5ndCIsImZhbWlseSI6Ikthcmxzb24ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUG8gVGVlbiIsImZhbWlseSI6IkxpbSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJMaW5jb2xuIiwiZmFtaWx5IjoiTWFjS2VuemllIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1hcmluYSIsImZhbWlseSI6Ik1vbnRyZXNvciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJWZXJhIEwuIiwiZmFtaWx5IjoiVHJhaW5lciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJHaXJlcyIsImZhbWlseSI6IlVzdXAifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiS2Vkb25nIiwiZmFtaWx5IjoiWWluIn19XSwiaWRzIjp7Imlzc24iOlsiMTA0Mi04Mjc1IiwiMjM3Ny02MTdYIl0sImlzYm4iOltdLCJkb2kiOiIxMC41NjcwL29jZWFub2cuMjAxNy4xMTEiLCJtYWciOiIyNTk3NjY0NzI3Iiwib3BlbmFsZXgiOiJXMjU5NzY2NDcyNyJ9LCJ0aXRsZSI6Ikdsb2JhbEhBQjogQSBOZXcgUHJvZ3JhbSB0byBQcm9tb3RlIEludGVybmF0aW9uYWwgUmVzZWFyY2gsIE9ic2VydmF0aW9ucywgYW5kIE1vZGVsaW5nIG9mIEhhcm1mdWwgQWxnYWwgQmxvb21zIGluIEFxdWF0aWMgU3lzdGVtcyIsImFic3RyYWN0IjoiQmVyZGFsZXQsIEVsaXNhIC4uLiBldCBhbC4tLSBTcGVjaWFsIGlzc3VlIG9uIEludGVybmF0aW9uYWwgQ29vcGVyYXRpb24gaW4gSGFybWZ1bCBBbGdhbCBCbG9vbSBTY2llbmNlLi0tIDEyIHBhZ2VzLCA2IGZpZ3VyZXMiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuNTY3MC9vY2Vhbm9nLjIwMTcuMTExIiwicmVzb3VyY2VVcmwiOiJodHRwczovL3Rvcy5vcmcvb2NlYW5vZ3JhcGh5L2Fzc2V0cy9kb2NzLzMwLTFfYmVyZGFsZXQzLnBkZiIsImlzc3VlZERhdGUiOiIyMDE3LTAzLTAxIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiT2NlYW5vZ3JhcGh5IiwiaXNzdWUiOjEsInZvbHVtZSI6MzAsImltcGFjdEZhY3RvciI6IjIuOTMifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjo3MCwibGFzdFBhZ2UiOjgxfX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6Ik9jZWFub2dyYXBoeSBTb2NpZXR5In0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjoyNiwiY2l0ZWRCeUNvdW50IjozMn19LCJxdW90ZSI6IlRoZSAgZWNvbm9taWMgZWZmZWN0cyBvZiBIQUJzIGFyaXNlIGZyb20gcHViLWxpYyBoZWFsdGggY29zdHMsIGNvbW1lcmNpYWwgZmlzaGVyeSBhbmQgIGFxdWFjdWx0dXJlIG9wZXJhdGlvbiBjbG9zdXJlcyBhbmQgZmlzaCAga2lsbHMsIHBvc3NpYmxlIG1lZGl1bS0gYW5kIGxvbmctdGVybSAgZGVjbGluZXMgaW4gY29hc3RhbCBhbmQgbWFyaW5lIHJlY3JlYXRpb24gIGFuZCB0b3VyaXNtLCBhbmQgdGhlIGNvc3RzIG9mIGluc3VyYW5jZSwgIG1vbml0b3JpbmcsIG1hbmFnZW1lbnQsIGFuZCBtaXRpZ2F0aW9uICAoTW9yZ2FuIGV0IGFsLiwgMjAxMCkuIEFnZ3JlZ2F0aW5nIGVjby1ub21pYyBlZmZlY3Rz4oCUYm90aCB3aXRoaW4gYW5kIGFjcm9zcyAgdGhlc2UgY2F0ZWdvcmllc%2BKAlGNhbiBhbHNvIGJlIHByb2JsZW0tYXRpYyB3aXRoIGF2YWlsYWJsZSBkYXRhIGFuZCBjdXJyZW50bHkgdXNlZCAgbWV0aG9kb2xvZ2llcyAoSG9hZ2xhbmQgZXQgYWwuLCAyMDAyKS4gIEFuIGV4YW1wbGUgb2YgYW4gZWNvbm9taWMgZXZhbHVhdGlvbiAgb2YgSEFCcyBhdCBhIG5hdGlvbmFsIGxldmVsIGlzIGdpdmVuIGJ5ICBBbmRlcnNvbiBldCBhbC4gKDIwMDApLCB3aG8gZXN0aW1hdGVkIHRoZSAgZWNvbm9taWMgZWZmZWN0cyBvZiBIQUJzIGluIHRoZSBVbml0ZWQgIFN0YXRlcyB0byBiZSAkMTAwIG1pbGxpb24gcGVyIHllYXIgKGFzIGxhdGVyICB0cmFuc2xhdGVkIGludG8gMjAxMiBkb2xsYXJzKSwgYnJva2VuIGRvd24gIGFzIGZvbGxvd3M6IDQ1JSBmb3IgcHVibGljIGhlYWx0aCBjb3N0cywgIDM3JSBmb3IgdGhlIGNvc3RzIG9mIGNsb3N1cmVzIGFuZCBsb3NzZXMgIGV4cGVyaWVuY2VkIGJ5IGNvbW1lcmNpYWwgZmlzaGVyaWVzLCAgMTMlIGZyb20gdGhlIGltcGFjdCBvbiBsb3N0IHJlY3JlYXRpb24gIGFuZCB0b3VyaXNtIG9wcG9ydHVuaXRpZXMsIGFuZCA0JSBmcm9tIEZJR1VSRSA2LiBNYXNzaXZlIGZpc2ggbW9ydGFsaXRpZXMgaW4gSmFrYXJ0YSwgSW5kb25lc2lhLiBQaG90byBjcmVkaXRzOiBUdW1wYWsgU2lkYWJ1dGFyIChIQU41MykgbW9uaXRvcmluZyBhbmQgbWFuYWdlbWVudCBjb3N0cy4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjoxMS44MTk0NDQ2NTYzNzIwNywibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzQ5NDUyOTcyNDEyMSwiaGVpZ2h0IjoxLjcwMzI4MjU5NDY4MDc4NjF9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjoxMy41MjM5ODk2Nzc0MjkyLCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI2Ljg2NDM3NDE2MDc2NjYsImhlaWdodCI6MS43MDMyODI1OTQ2ODA3ODYxfX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6MTUuMjI4NTM1NjUyMTYwNjQ1LCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzMjAwMjI1ODMwMDc4LCJoZWlnaHQiOjEuNzAzMjgzNTQ4MzU1MTAyNX19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjE2LjkzMzA4MDY3MzIxNzc3MywibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzM2MzY4NTYwNzkxLCJoZWlnaHQiOjEuNzAzMjgzNTQ4MzU1MTAyNX19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjE4LjYzNzYyNjY0Nzk0OTIyLCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzNTI5NDM0MjA0MTAxNiwiaGVpZ2h0IjoxLjcwMzI4MzU0ODM1NTEwMjV9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjoyMC4zNDIxNzA3MTUzMzIwMywibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzM5ODU5MDA4Nzg5MDYsImhlaWdodCI6MS43MDMyODM1NDgzNTUxMDI1fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6MjIuMDQ2NzE2NjkwMDYzNDc3LCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzMjAxMTc5NTA0Mzk0NSwiaGVpZ2h0IjoxLjcwMzI4MzU0ODM1NTEwMjV9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjoyMy43NTEyNjI2NjQ3OTQ5MjIsImxlZnQiOjY3LjI1NDg5ODA3MTI4OTA2LCJ3aWR0aCI6MjcuMjMyMDQyMzEyNjIyMDcsImhlaWdodCI6MS43MDMyODM1NDgzNTUxMDI1fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6MjUuNDU1ODA4NjM5NTI2MzY3LCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI2Ljg2MzA2NzYyNjk1MzEyNSwiaGVpZ2h0IjoxLjcwMzI4MzU0ODM1NTEwMjV9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjoyNy4xNjAzNTI3MDY5MDkxOCwibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzU0MzM1Nzg0OTEyMSwiaGVpZ2h0IjoxLjcwMzI4MzU0ODM1NTEwMjV9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjoyOC44NjQ4OTg2ODE2NDA2MjUsImxlZnQiOjY3LjI1NDg5ODA3MTI4OTA2LCJ3aWR0aCI6MjYuODY0Mzk1MTQxNjAxNTYyLCJoZWlnaHQiOjEuNzAzMjgzNTQ4MzU1MTAyNX19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjMwLjU2OTQ0Mjc0OTAyMzQzOCwibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzQ5MjQzMTY0MDYyNSwiaGVpZ2h0IjoxLjcwMzI4MzU0ODM1NTEwMjV9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjozMi4yNzM5OTA2MzExMDM1MTYsImxlZnQiOjY3LjI1NDg5ODA3MTI4OTA2LCJ3aWR0aCI6MjcuMjM0Nzc1NTQzMjEyODksImhlaWdodCI6MS43MDMyODM1NDgzNTUxMDI1fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6MzMuOTc4NTM0Njk4NDg2MzMsImxlZnQiOjY3LjI1NDg5ODA3MTI4OTA2LCJ3aWR0aCI6MjcuMjM0MzM2ODUzMDI3MzQ0LCJoZWlnaHQiOjEuNzAzMjg1NDU1NzAzNzM1NH19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjM1LjY4MzA3ODc2NTg2OTE0LCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzMjE2MDU2ODIzNzMwNSwiaGVpZ2h0IjoxLjcwMzI4NTQ1NTcwMzczNTR9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjozNy4zODc2MjI4MzMyNTE5NSwibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzM2NjU0NjYzMDg1OTQsImhlaWdodCI6MS43MDMyODU0NTU3MDM3MzU0fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6MzkuMDkyMTcwNzE1MzMyMDMsImxlZnQiOjY3LjI1NDg5ODA3MTI4OTA2LCJ3aWR0aCI6MjcuMjM2NTc5ODk1MDE5NTMsImhlaWdodCI6MS43MDMyODU0NTU3MDM3MzU0fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6NDAuNzk2NzE0NzgyNzE0ODQ0LCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzNjcxNzIyNDEyMTA5NCwiaGVpZ2h0IjoxLjcwMzI4NTQ1NTcwMzczNTR9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo0Mi41MDEyNjI2NjQ3OTQ5MiwibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzM2MjczMTkzMzU5MzgsImhlaWdodCI6MS43MDMyODU0NTU3MDM3MzU0fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6NDQuMjA1ODA2NzMyMTc3NzM0LCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzMDU0NTA0Mzk0NTMxMiwiaGVpZ2h0IjoxLjcwMzI4NTQ1NTcwMzczNTR9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo0NS45MTAzNTA3OTk1NjA1NSwibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzg1NDQ0NjQxMTEzMjgsImhlaWdodCI6MS43MDMyODU0NTU3MDM3MzU0fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6NDcuNjE0ODk0ODY2OTQzMzYsImxlZnQiOjY3LjI1NDg5ODA3MTI4OTA2LCJ3aWR0aCI6MjcuMjM0MTQ2MTE4MTY0MDYyLCJoZWlnaHQiOjEuNzAzMjg1NDU1NzAzNzM1NH19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjQ5LjMxOTQ0Mjc0OTAyMzQ0LCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzNTk0MDkzMzIyNzU0LCJoZWlnaHQiOjEuNzAzMjg1NDU1NzAzNzM1NH19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjUxLjAyMzk5MDYzMTEwMzUxNiwibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzY5MDc5NTg5ODQzNzUsImhlaWdodCI6MS43MDMyODU0NTU3MDM3MzU0fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6OTIuMjc4NzcwNDQ2Nzc3MzQsImxlZnQiOjguODIzNTI5MjQzNDY5MjM4LCJ3aWR0aCI6NTYuMjYzOTE2MDE1NjI1LCJoZWlnaHQiOjEuMjA5MDg5NTE3NTkzMzgzOH19LHsicGFnZU51bWJlciI6OSwiYmJveCI6eyJ0b3AiOjUuMDAxMjYyNjY0Nzk0OTIyLCJsZWZ0Ijo1Ljg4MjM1MjgyODk3OTQ5Miwid2lkdGgiOjI3LjIzNTI5NDM0MjA0MTAxNiwiaGVpZ2h0IjoxLjcwMzI4MjU5NDY4MDc4NjF9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI3OCJ9fV0%3D.

Challenges include knowledge erosion among youth, land encroachments, and validation amid data scarcity, yet platforms like DataStream offer scalable solutions (Simon et al., 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkRhbmEgRi4iLCJmYW1pbHkiOiJTaW1vbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJHYWJyaWVsIiwiZmFtaWx5IjoiTXVub3oifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUXVvYyBUdWMiLCJmYW1pbHkiOiJEaW5oIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlN1bmcgVm8iLCJmYW1pbHkiOiJEdXkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiS2F0IiwiZmFtaWx5IjoiS2F2YW5hZ2gifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUm9iZXJ0IEouIiwiZmFtaWx5IjoiU21pdGgifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQmFycnkiLCJmYW1pbHkiOiJIdXNrIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlPDqWJhc3RpZW4iLCJmYW1pbHkiOiJTYXV2w6kifX1dLCJpZHMiOnsiaXNzbiI6WyIyMDU3LTQ5OTEiXSwiaXNibiI6W10sImRvaSI6IjEwLjUzMzQvY3N0cC42NTUiLCJvcGVuYWxleCI6Ilc0MzkwNDA1NzM0In0sInRpdGxlIjoiQWRvcHQgYSBMYWtlOiBTdWNjZXNzZnVsbHkgVHJhY2tpbmcgSGFybWZ1bCBDeWFub2JhY3RlcmlhbCBCbG9vbXMgaW4gQ2FuYWRpYW4gU3VyZmFjZSBXYXRlcnMgVGhyb3VnaCBDaXRpemVuIFNjaWVuY2UiLCJhYnN0cmFjdCI6IlRoZSBwcm9saWZlcmF0aW9uIG9mIGhhcm1mdWwgd2F0ZXJib3JuZSBjeWFub2JhY3RlcmlhbCBhbGdhbCBibG9vbXMsIHNvbWUgb2Ygd2hpY2ggY2FuIHByb2R1Y2UgcG90ZW50IHRveGlucywgcG9zZXMgc2V2ZXJlIHJpc2tzIHRvIGVudmlyb25tZW50YWwgYW5kIGh1bWFuIGhlYWx0aC4gQWNhZGVtaWMgYW5kIGdvdmVybm1lbnRhbCBtb25pdG9yaW5nIGVmZm9ydHMgbWF5IGJlIGNvbnN0cmFpbmVkIGJ5IGJ1ZGdldCwgdGltZSwgYW5kIHN0YWZmLCBhbmQgdGh1cyBtaXNzIG90aGVyd2lzZSBzaWduaWZpY2FudCBwb2xsdXRpb24gZXZlbnRzLiBIZXJlLCB3ZSByZXBvcnQgb24gdGhlIGltcGxlbWVudGF0aW9uIG9mIGEgY2l0aXplbiBzY2llbmNlIHByb2plY3QgdG8gdHJhY2sgaGFybWZ1bCBjeWFub2JhY3RlcmlhbCBibG9vbXMgaW4gbGFrZXMgYW5kIHdhdGVyd2F5cyBhY3Jvc3MgQ2FuYWRhLiBUaHJvdWdoIGJvdGggY3Jvd2Rzb3VyY2luZyBhbmQgY3Jvd2RmdW5kaW5nLCB0aGUgQWRvcHQgYSBMYWtlIChBZG9wdCBhIExha2UgMjAyMikgY2FtcGFpZ24gYWltZWQgdG8gZG9jdW1lbnQgdGhlIHBvdGVudGlhbCBwcmVzZW5jZSBvZiBjeWFub2JhY3RlcmlhIGFuZCB0b3hpbnMgd2l0aCB0aGUgYXNzaXN0YW5jZSBvZiBwYXJ0aWNpcGFudHMsIHRodXMgaW1wcm92aW5nIHB1YmxpYyBhd2FyZW5lc3Mgb2YgdGhlIGlzc3VlIG9mIHdhdGVyIHF1YWxpdHkgcHJlc2VydmF0aW9uLiBVc2luZyBzb2NpYWwgbWVkaWEsIHBhcnRpY2lwYW50cyB3ZXJlIGVuY291cmFnZWQgdG8gcGFydGljaXBhdGUgaW4gdGhlIGluaXRpYXRpdmUgYnkgY29sbGVjdGluZyBzYW1wbGVzIGR1cmluZyBhIGJsb29tIGZyb20gYSBuZWFyYnkgcG9uZCBvciBieSBtYWtpbmcgYSBmaW5hbmNpYWwgY29udHJpYnV0aW9uIHRvIHN1cHBvcnQgdGhlIGluaXRpYXRpdmUuIEFkb3B0IGEgTGFrZSBiZW5lZml0dGVkIGZyb20gdGhlIGFuYWx5dGljYWwgcGxhdGZvcm0gb2YgQWxnYWwgQmxvb21zIFRyZWF0bWVudCwgUmlzayBBc3Nlc3NtZW50LCBQcmVkaWN0aW9ucywgYW5kIFByZXZlbnRpb24gKEFUUkFQUCksIGEgcmVzZWFyY2ggcHJvamVjdCBmb2N1c2VkIG9uIHRoZSBwcmVkaWN0aW9uIGFuZCBtYW5hZ2VtZW50IG9mIGhhcm1mdWwgY3lhbm9iYWN0ZXJpYWwgYmxvb21zLiBUaGUgcHJlc2VuY2Ugb2YgY3lhbm90b3hpbnMsIHdoaWNoIGNhbiBjb25maXJtIHdoZXRoZXIgYSBsYWtlIGhhcyBhIHRveGljIGJsb29tLCB3YXMgZGV0ZXJtaW5lZCB0aHJvdWdoIGhpZ2gtcmVzb2x1dGlvbiBtYXNzIHNwZWN0cm9tZXRyeSBhbmFseXNlcy4gVGhpcyBwYXBlciBwcmVzZW50cyBhbiBvdmVydmlldyBvZiB0aGUgaW1wbGVtZW50YXRpb24gb2YgdGhlIEFkb3B0IGEgTGFrZSBpbml0aWF0aXZlLCB0aGUgY2FtcGFpZ27igJlzIHN0YXR1cywgYW5kIHRoZSBsZXNzb25zIGxlYXJuZWQsIGFuZCBpdCBhcmd1ZXMgdGhlIGltcG9ydGFuY2Ugb2YgY29udGludWFsIG1vbml0b3Jpbmcgb2YgY3lhbm9iYWN0ZXJpYWwgYmxvb21zLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC41MzM0L2NzdHAuNjU1IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3N0b3JhZ2UuZ29vZ2xlYXBpcy5jb20vam5sLXVwLWotY3N0cC1maWxlcy9qb3VybmFscy8xL2FydGljbGVzLzY1NS82NThlOWQ3ZDQ2YTBjLnBkZiIsImlzc3VlZERhdGUiOiIyMDIzLTEyLTI5IiwiYWNjZXNzZWREYXRlIjoiMjAyNS0wOCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiQ2l0aXplbiBTY2llbmNlIFRoZW9yeSBhbmQgUHJhY3RpY2UiLCJpc3N1ZSI6MSwidm9sdW1lIjo4LCJpbXBhY3RGYWN0b3IiOiIzLjEyIn0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6NjYsImxhc3RQYWdlIjo2Nn19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJVYmlxdWl0eSBQcmVzcyJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MjcsImNpdGVkQnlDb3VudCI6Mn19LCJxdW90ZSI6IkluIHRoZSBVbml0ZWQgIFN0YXRlcywgYSB2b2x1bnRlZXIgbW9uaXRvcmluZyBwcm9ncmFtIGRldmVsb3BlZCBieSB0aGUgIFdhYmFzaCBSaXZlciBFbmhhbmNlbWVudCBDb3Jwb3JhdGlvbiBoZWxwZWQgdG8gaWRlbnRpZnkgIHdhdGVyLXF1YWxpdHkgcHJpb3JpdGllcyBhY3Jvc3MgdGhlIHdhdGVyc2hlZCAoTXVlbmljaCAgZXQgYWwuIDIwMTYpLiBJbiBDYW5hZGEsIG9yZ2FuaXphdGlvbnMgYXJlIGNvbW1pdHRlZCAgdG8gbW9uaXRvciBhbmQgcHJvdGVjdCB3YXRlciB0aHJvdWdoIGNyb3dkc291cmNlZCAgbW9uaXRvcmluZyBhbmQga25vd2xlZGdlIHRyYW5zZmVyIGluaXRpYXRpdmVzIChHM0UgIDIwMjI7IE90dGF3YSBSaXZlcmtlZXBlciAyMDIzOyBPdXIgTGl2aW5nIFdhdGVycyAyMDIyOyAgV2F0ZXIgUmFuZ2VycyAyMDIyOyBaaGFvLCBMaXUgYW5kIFdlaSAyMDIwKS5UaGVzZSAgZWZmb3J0cyBhcmUgY29tcGxlbWVudGVkIGJ5IG9wZW4gZnJlZSBwbGF0Zm9ybXMgc3VjaCAgYXMgRGF0YVN0cmVhbSwgd2hlcmUgd2F0ZXIgcXVhbGl0eSBkYXRhIGNvbGxlY3RlZCBmcm9tICB2YXJpb3VzIGNvbW11bml0eS1nYXRoZXJlZCBwcm9ncmFtcyBpcyBjdXJhdGVkLCBzdG9yZWQsICBhbmQgc2hhcmVkIHdpdGggdGhlIHB1YmxpYyAoVGhlIEdvcmRvbiBGb3VuZGF0aW9uIDIwMjMpLiBHaXZlbiB0aGUgY3JpdGljYWwgaW1wb3J0YW5jZSBvZiB3YXRlciByZXNvdXJjZXMgYW5kIHRoZWlyIGRpdmVyc2UgdXNlcywgaW5jbHVkaW5nIGRvbWVzdGljIHdlbGxzLCBtdW5pY2lwYWwgZHJpbmtpbmcgd2F0ZXIgcHJvZHVjdGlvbiwgZmlzaGluZywgcmVjcmVhdGlvbmFsIGFjdGl2aXRpZXMsICBhbmQgY3JvcCBpcnJpZ2F0aW9uLCBpdCBpcyBlc3NlbnRpYWwgdG8gcHJvdGVjdCBhbmQgbW9uaXRvciAgdGhlbS4gTGFrZXMsIG9mdGVuIHVzZWQgZm9yIHJlY3JlYXRpb25hbCBhY3Rpdml0aWVzIGFuZCBhcyAgZHJpbmtpbmcgd2F0ZXIgc291cmNlcywgYXJlIHN1c2NlcHRpYmxlIHRvIGJsb29tcyBvZiBhbGdhZSAgYW5kIGN5YW5vYmFjdGVyaWEsIHBhcnRpY3VsYXJseSBpbiB3YXJtLCBzbG93LW1vdmluZywgYW5kICBudXRyaWVudC1yaWNoIHdhdGVycy4gVGhlc2UgYmxvb21zIGNhbiBtYW5pZmVzdCBhcyBmb2FtLCAgc2N1bSwgbWF0cywgb3IgcGFpbnQtbGlrZSBjb2F0aW5ncyBvbiB0aGUgd2F0ZXLigJlzIHN1cmZhY2UgIGFuZCBjYW4gcHJvZHVjZSBjeWFub3RveGlucyB0aGF0IHBvc2UgaGVhbHRoIHJpc2tzIHRvICBodW1hbnMgYW5kIHdpbGRsaWZlIChGaWd1cmUgMSkuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MTIuNDY0MDcyMjI3NDc4MDI3LCJsZWZ0Ijo1MS44NTQ4ODg5MTYwMTU2MjUsIndpZHRoIjozOC43ODY3NTQ2MDgxNTQzLCJoZWlnaHQiOjEuMzU2ODE4NDM3NTc2Mjk0fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MTQuMTA0OTc4NTYxNDAxMzY3LCJsZWZ0Ijo1MS44NTM0MTY0NDI4NzEwOTQsIndpZHRoIjozOC43ODk2OTE5MjUwNDg4MywiaGVpZ2h0IjoxLjM1NjgxODQzNzU3NjI5NH19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjE1Ljc0NTg4Mzk0MTY1MDM5LCJsZWZ0Ijo1MS44NTM0MTY0NDI4NzEwOTQsIndpZHRoIjozOC43ODY3MzkzNDkzNjUyMzQsImhlaWdodCI6MS4zNTY4MTg0Mzc1NzYyOTR9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoxNy4zODY3ODkzMjE4OTk0MTQsImxlZnQiOjUxLjg1MzQxNjQ0Mjg3MTA5NCwid2lkdGgiOjM4Ljc4ODIxNTYzNzIwNzAzLCJoZWlnaHQiOjEuMzU2ODE4NDM3NTc2Mjk0fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MTkuMDI3Njk0NzAyMTQ4NDM4LCJsZWZ0Ijo1MS44NTM0MTY0NDI4NzEwOTQsIndpZHRoIjozOC43ODgyMTU2MzcyMDcwMywiaGVpZ2h0IjoxLjM1NjgxODQzNzU3NjI5NH19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjIwLjY2ODYwMDA4MjM5NzQ2LCJsZWZ0Ijo1MS44NTM0MTY0NDI4NzEwOTQsIndpZHRoIjozOC43ODUyNzQ1MDU2MTUyMzQsImhlaWdodCI6MS4zNTY4MTg0Mzc1NzYyOTR9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyMi4zMDk1MDU0NjI2NDY0ODQsImxlZnQiOjUxLjg1MzQxNjQ0Mjg3MTA5NCwid2lkdGgiOjM4Ljc4ODIzNDcxMDY5MzM2LCJoZWlnaHQiOjEuMzU2ODE4NDM3NTc2Mjk0fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MjMuOTUwNDEwODQyODk1NTA4LCJsZWZ0Ijo1MS44NTM0MTI2MjgxNzM4Mywid2lkdGgiOjM4Ljc4MzgzMjU1MDA0ODgzLCJoZWlnaHQiOjEuMzU2ODE4NDM3NTc2Mjk0fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MjUuNTkxMzE2MjIzMTQ0NTMsImxlZnQiOjUxLjg0OTAwNjY1MjgzMjAzLCJ3aWR0aCI6MzguNzkxMTcyMDI3NTg3ODksImhlaWdodCI6MS4zNTY4MTg0Mzc1NzYyOTR9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyNy4yMzIyMjE2MDMzOTM1NTUsImxlZnQiOjUxLjg1MTk0MDE1NTAyOTMsIndpZHRoIjozOC43OTI2MTc3OTc4NTE1NiwiaGVpZ2h0IjoxLjM1NjgxODQzNzU3NjI5NH19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI4Ljg3MzEyNjk4MzY0MjU3OCwibGVmdCI6NTEuODUxOTQwMTU1MDI5Mywid2lkdGgiOjM4Ljc4Mzc3NTMyOTU4OTg0NCwiaGVpZ2h0IjoxLjM1NjgxODQzNzU3NjI5NH19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMwLjUxNDAzNDI3MTI0MDIzNCwibGVmdCI6NTEuODUxOTQwMTU1MDI5Mywid2lkdGgiOjM4Ljc4ODIzMDg5NTk5NjA5NCwiaGVpZ2h0IjoxLjM1NjgxODQzNzU3NjI5NH19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMyLjE1NDkzNzc0NDE0MDYyNSwibGVmdCI6NTEuODUxOTQwMTU1MDI5Mywid2lkdGgiOjM4LjU4NTI4OTAwMTQ2NDg0NCwiaGVpZ2h0IjoxLjM1NjgxODQzNzU3NjI5NH19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMzLjc5NTg0NTAzMTczODI4LCJsZWZ0Ijo1My43MDQ4Nzk3NjA3NDIxOSwid2lkdGgiOjM2LjkzNTI3OTg0NjE5MTQwNiwiaGVpZ2h0IjoxLjM1NjgxODQzNzU3NjI5NH19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjM1LjQzNjc1MjMxOTMzNTk0LCJsZWZ0Ijo1MS44NTE5NDAxNTUwMjkzLCJ3aWR0aCI6MzguNzg5NzA3MTgzODM3ODksImhlaWdodCI6MS4zNTY4MTg0Mzc1NzYyOTR9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNy4wNzc2NjM0MjE2MzA4NiwibGVmdCI6NTEuODUxOTQwMTU1MDI5Mywid2lkdGgiOjM4Ljc4Njc2NjA1MjI0NjA5NCwiaGVpZ2h0IjoxLjM1NjgxODQzNzU3NjI5NH19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjM4LjcxODU3MDcwOTIyODUxNiwibGVmdCI6NTEuODUxOTQwMTU1MDI5Mywid2lkdGgiOjM4Ljc4MzgxMzQ3NjU2MjUsImhlaWdodCI6MS4zNTY4MTg0Mzc1NzYyOTR9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0MC4zNTk0ODE4MTE1MjM0NCwibGVmdCI6NTEuODUxOTQwMTU1MDI5Mywid2lkdGgiOjM4Ljc4Njc3MzY4MTY0MDYyNSwiaGVpZ2h0IjoxLjM1NjgxODQzNzU3NjI5NH19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQyLjAwMDM5MjkxMzgxODM2LCJsZWZ0Ijo1MS44NTE5NDAxNTUwMjkzLCJ3aWR0aCI6MzguNzg1MzAxMjA4NDk2MDk0LCJoZWlnaHQiOjEuMzU2ODE4NDM3NTc2Mjk0fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDMuNjQxMzAwMjAxNDE2MDE2LCJsZWZ0Ijo1MS44NTE5NDAxNTUwMjkzLCJ3aWR0aCI6MzguNzkxMTUyOTU0MTAxNTYsImhlaWdodCI6MS4zNTY4MTg0Mzc1NzYyOTR9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0NS4yODIyMDc0ODkwMTM2NywibGVmdCI6NTEuODUxOTQwMTU1MDI5Mywid2lkdGgiOjM4Ljc4OTcwNzE4MzgzNzg5LCJoZWlnaHQiOjEuMzU2ODE4NDM3NTc2Mjk0fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDYuOTIzMTE4NTkxMzA4NTk0LCJsZWZ0Ijo1MS44NTE5NDAxNTUwMjkzLCJ3aWR0aCI6MzguNzkyNjI5MjQxOTQzMzYsImhlaWdodCI6MS4zNTY4MTg0Mzc1NzYyOTR9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0OC41NjQwMjk2OTM2MDM1MTYsImxlZnQiOjUxLjg1MTk0MDE1NTAyOTMsIndpZHRoIjozOC43ODUyNzgzMjAzMTI1LCJoZWlnaHQiOjEuMzU2ODE4NDM3NTc2Mjk0fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTAuMjA0OTM2OTgxMjAxMTcsImxlZnQiOjUxLjg1MTk0MDE1NTAyOTMsIndpZHRoIjozOC43ODY3NjYwNTIyNDYwOTQsImhlaWdodCI6MS4zNTY4MTg0Mzc1NzYyOTR9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI2NyJ9fV0%3D. By institutionalizing TEK via state wetland authorities, India can democratize HAB mitigation, blending bottom-up wisdom with top-down policies to avert tourism slumps and real estate devaluation (Devlin et al., 2020; Sampat et al., 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjEsInN1YnR5cGUiOnt9LCJjb250cmlidXRvcnMiOlt7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBZGFtIFQuIiwiZmFtaWx5IjoiRGV2bGluIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkppYXlpIiwiZmFtaWx5IjoiUGFuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik0uIE1hbmp1ciIsImZhbWlseSI6IlNoYWgifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQWxpeXUiLCJmYW1pbHkiOiJOdWh1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Iktlbm5ldGggVy4iLCJmYW1pbHkiOiJDdW1taW5zIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkNsYXVkaW8gVmFsZG92aW5vcyIsImZhbWlseSI6IlphcmdlcyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQYWJsbyIsImZhbWlseSI6IkZpZXJybyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJWaXZpYW5hIiwiZmFtaWx5IjoiT2xtb3MifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQWJkdWwtUmFobWFuIiwiZmFtaWx5IjoiRGlyaXN1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkpvaG4gT3ZpZSIsImZhbWlseSI6Ik9sb211a29ybyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJOYXRpdmlkYWRlIiwiZmFtaWx5IjoiVmllaXJhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFuYSIsImZhbWlseSI6IlNvYXJlcyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJDYXJsb3MiLCJmYW1pbHkiOiJQaW5oZWlybyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNYXR0aGV3IEYuIiwiZmFtaWx5IjoiQmluZ2hhbSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJKYXNvbiIsImZhbWlseSI6Iktpbm5lbGwifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiV3UiLCJmYW1pbHkiOiJCbyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJKaW5tdSIsImZhbWlseSI6IlpoYW5nIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllpbmRpIiwiZmFtaWx5IjoiWmhhbyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJIdW1iZXJ0byBBbHZlcyIsImZhbWlseSI6IkJhcmJvc2EifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiRnJhbmtsaW4iLCJmYW1pbHkiOiJQYXJlZGVz4oCQVHJlam8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiVCBMYWtzaG1pIiwiZmFtaWx5IjoiS3VtYXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiS2lyYW4iLCJmYW1pbHkiOiJUaGFrdXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQ2F0YXJpbmEiLCJmYW1pbHkiOiJCdXJpdGkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2NvdHQgQS4iLCJmYW1pbHkiOiJXZWxscyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJPc2NhciBPbW9uZGkiLCJmYW1pbHkiOiJEb25kZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJDYXJlbiBOYXZhbGlhIiwiZmFtaWx5IjoiQXRhbGl0c2EifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiRC4iLCJmYW1pbHkiOiJTYW5pIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNhbXBzb24iLCJmYW1pbHkiOiJBaG1lZCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBbWFuaW5nIiwiZmFtaWx5IjoiS3dha3UifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiR2V2b3JnIiwiZmFtaWx5IjoiQWRqZWkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2F0YXJ1cGEiLCJmYW1pbHkiOiJTaW1vbnlhbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTbmlnZGhhIiwiZmFtaWx5IjoiR2hvc2gifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUHJhc2FubmEiLCJmYW1pbHkiOiJDaGF0dGVyamVlIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNoaXZhIiwiZmFtaWx5IjoiR2hvcmEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQmVsZGEiLCJmYW1pbHkiOiJQcmFzYWQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiS2V0aGxhdGxvZ2lsZSIsImZhbWlseSI6Ik1vc2VwZWxlIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ill1c3VmIiwiZmFtaWx5IjoiU3VsZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJVaXLDoSIsImZhbWlseSI6Ik9saXZlaXJhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1hcmlhIEpvw6NvIiwiZmFtaWx5IjoiVmllaXJhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkZsZXVyIiwiZmFtaWx5IjoiRGUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTWluIiwiZmFtaWx5IjoiTWF4In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikdldm9yZyIsImZhbWlseSI6IlNpbW9ueWFuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNhdGFydXBhIiwiZmFtaWx5IjoiR2hvc2gifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU25pZ2RoYSIsImZhbWlseSI6IkNoYXR0ZXJqZWUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUHJhc2FubmEiLCJmYW1pbHkiOiJQcmFzYWQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2F0YXJ1cGEiLCJmYW1pbHkiOiJHaG9zaCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJHaG9yYSIsImZhbWlseSI6IlByYXNhZCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQcmFzYW5uYSIsImZhbWlseSI6IlBhbCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLd2FrdSIsImZhbWlseSI6IkFnb2R6byJ9fV0sImlkcyI6eyJpc3NuIjpbXSwiaXNibiI6W10sImRvaSI6IjEwLjU3NzIvaW50ZWNob3Blbi44NzQ2MyIsIm1hZyI6IjMxMjg1ODc0NjkiLCJvcGVuYWxleCI6IlczMTI4NTg3NDY5In0sInRpdGxlIjoiSW5sYW5kIFdhdGVycyAtIER5bmFtaWNzIGFuZCBFY29sb2d5IiwiYWJzdHJhY3QiOiJJbmxhbmQgd2F0ZXJzLCBsYWtlcywgcml2ZXJzLCBhbmQgdGhlaXIgY29ubmVjdGVkIHdldGxhbmRzIGFyZSB0aGUgbW9zdCBpbXBvcnRhbnQgYW5kIHRoZSBtb3N0IHZ1bG5lcmFibGUgc291cmNlcyBvZiBmcmVzaHdhdGVyIG9uIHRoZSBwbGFuZXQuIFRoZSBlY29sb2d5IG9mIHRoZXNlIHN5c3RlbXMgaW5jbHVkZXMgYmlvbG9neSBhcyB3ZWxsIGFzIGh1bWFuIHBvcHVsYXRpb25zIGFuZCBjaXZpbGl6YXRpb24uIElubGFuZCB3YXRlcnMgYW5kIHdldGxhbmRzIGFyZSBoaWdobHkgc3VzY2VwdGlibGUgdG8gY2hlbWljYWwgYW5kIGJpb2xvZ2ljYWwgcG9sbHV0YW50cyBmcm9tIG5hdHVyYWwgb3IgaHVtYW4gc291cmNlcywgY2hhbmdlcyBpbiB3YXRlcnNoZWQgZHluYW1pY3MgZHVlIHRvIHRoZSBlc3RhYmxpc2htZW50IG9mIGRhbXMgYW5kIHJlc2Vydm9pcnMsIGFuZCBsYW5kIHVzZSBjaGFuZ2VzIGZyb20gYWdyaWN1bHR1cmUgYW5kIGluZHVzdHJ5LiBUaGlzIGJvb2sgcHJvdmlkZXMgYSBjb21wcmVoZW5zaXZlIHJldmlldyBvZiBpc3N1ZXMgaW52b2x2aW5nIGlubGFuZCB3YXRlcnMgYW5kIGRpc2N1c3NlcyBtYW55IHdvcmxkd2lkZSBpbmxhbmQgd2F0ZXIgc3lzdGVtcy4gVGhlIG1haW4gdG9waWNzIG9mIHRoaXMgdGV4dCBhcmUgd2F0ZXIgcXVhbGl0eSBpbnZlc3RpZ2F0aW9uLCBhbmFseXNlcyBvZiB0aGUgZWNvbG9neSBvZiBpbmxhbmQgd2F0ZXIgc3lzdGVtcywgcmVtb3RlIHNlbnNpbmcgb2JzZXJ2YXRpb24gYW5kIG51bWVyaWNhbCBtb2RlbGluZyBtZXRob2RzLCBhbmQgYmlvZGl2ZXJzaXR5IGludmVzdGlnYXRpb25zLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC41NzcyL2ludGVjaG9wZW4uODc0NjMiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vbXRzLmludGVjaG9wZW4uY29tL3N0b3JhZ2UvYm9va3MvOTY2MC9hdXRob3JzX2Jvb2svYXV0aG9yc19ib29rLnBkZiIsImlzc3VlZERhdGUiOiIyMDIwLTA0LTAxIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiSW50ZWNoT3BlbiBlQm9va3MiLCJpbXBhY3RGYWN0b3IiOiIxLjAxIn0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkludGVjaE9wZW4ifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjMzOCwiY2l0ZWRCeUNvdW50IjoyN319LCJxdW90ZSI6IkFkZGl0aW9uYWxseSwgc2luY2UgYWZmZWN0ZWQgYnVzaW5lc3Nlc%2BKAmSB2YWx1ZXMgIGFyZSBtb3N0IGxpa2VseSB0aWVkIHRvIHRoZWlyIGFzc2V0cyBhbmQgdGhlIHJlYWwgZXN0YXRlIHRoZXkgb2NjdXB5IChmb3IgZXhhbXBsZSwgIGEgbWFyaW5hIGlzIG5vdCBlYXNpbHkgY29udmVydGVkIHRvIHNvbWUgb3RoZXIgdXNlKSwgb24tZ29pbmcgYmFsYW5jZSBzaGVldCBlZmZlY3RzICB3b3VsZCB1bHRpbWF0ZWx5IGxlYWQgdG8gcmVkdWN0aW9ucyBpbiBjb21tZXJjaWFsIHJlYWwgZXN0YXRlIHZhbHVlcy4gVGhlcmUgYXJlIG1hbnkgY2hhbGxlbmdlcyB0byB1bmRlcnN0YW5kaW5nIHRoZSBpbXBsaWNhdGlvbnMgb2YgY2hhbmdlcyBpbiAgdG91cmlzbSBmcm9tIEhBQnMuIFRoZSBjbGVhcmVzdCBjaGFsbGVuZ2Ugb2JzdHJ1Y3RpbmcgYSBwcmVjaXNlIGFuYWx5c2lzIG9mIHRoZXNlICBpbXBhY3RzIGlzIGEgbGFjayBvZiBkYXRhIGVpdGhlciBvbiB0aGUgYW1vdW50IG9mIHRvdXJpc20gYXQgcmlzayBvciB0aGUgc3BlY2lmaWMgIGltcGFjdCBvZiBIQUJzIG9uIHRvdXJpc20uIEZvciBleGFtcGxlLCB3aGlsZSBjb3VudHktbGV2ZWwgZGF0YSBleGlzdHMgZm9yIHRvdGFsICBleHBlbmRpdHVyZXMgb24gdG91cmlzbSwgdGhpcyBpbmNsdWRlcyB0b3VyaXNtIHdoaWNoIHdvdWxkIG5vdCBiZSBpbnRlcnJ1cHRlZCBieSAgSEFCcyBvciBvdGhlciBkaXNjb3VyYWdpbmcgZmFjdG9ycy4gQW4gYWRkaXRpb25hbCBjaGFsbGVuZ2UgcmVsYXRlcyB0byB0aGUgZGlzdGluY3Rpb24gYmV0d2VlbiBlY29ub21pYyBiZW5lZml0cyAgKHdpbGxpbmduZXNzIHRvIHBheSkgYW5kIGVjb25vbWljIGltcGFjdHMgKGV4cGVuZGl0dXJlcyksIGFuZCB0aGUgbWVhc3VyZS1tZW50IG9mIHRoZSBlY29ub21pYyBiZW5lZml0cyB0aGF0IGFyaXNlIGZyb20gZWNvbm9taWMgaW1wYWN0cyAocHJvZml0cykuIEZvciAgZXhhbXBsZSwgY29uc2lkZXIgYSByZXN0YXVyYW50IG93bmVyIHdobyBsb3NlcyAkMTAsMDAwIGluIHJldmVudWUgYmVjYXVzZSBvZiBhICBIQUIuIFRoZSBvd25lcuKAmXMgd2lsbGluZ25lc3MgdG8gcGF5IHRvIHJlY292ZXIgdGhhdCByZXZlbnVlOyBpcyAocm91Z2hseSBzcGVha2luZykgIHRoZSBsb3N0IHByb2ZpdCBvbiB0aGF0IHJldmVudWUuIFRoaXMgaXMgbW9yZSBkaWZmaWN1bHQgdG8gaWRlbnRpZnkgdGhhbiBsb3N0IHJldmVudWUuICAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6ODksImJib3giOnsidG9wIjo1OS40NDI4NjcyNzkwNTI3MzQsImxlZnQiOjE0LjcyMjg1MzY2MDU4MzQ5Niwid2lkdGgiOjY4Ljc0ODAwMTA5ODYzMjgxLCJoZWlnaHQiOjEuOTA1MDAwODA1ODU0Nzk3NH19LHsicGFnZU51bWJlciI6ODksImJib3giOnsidG9wIjo2MS4wNzEwNzU0Mzk0NTMxMjUsImxlZnQiOjE0LjcyMjg1MzY2MDU4MzQ5Niwid2lkdGgiOjY2LjM5NjM0NzA0NTg5ODQ0LCJoZWlnaHQiOjEuOTA1MDAwODA1ODU0Nzk3NH19LHsicGFnZU51bWJlciI6ODksImJib3giOnsidG9wIjo2Mi42OTkyNzU5NzA0NTg5ODQsImxlZnQiOjE0LjcyMjg1MzY2MDU4MzQ5Niwid2lkdGgiOjY5LjI2OTMwOTk5NzU1ODYsImhlaWdodCI6MS45MDUwMDA4MDU4NTQ3OTc0fX0seyJwYWdlTnVtYmVyIjo4OSwiYmJveCI6eyJ0b3AiOjY0LjMyNzQ4NDEzMDg1OTM4LCJsZWZ0IjoxNC43MjI4NTM2NjA1ODM0OTYsIndpZHRoIjo1Ny40MDA1MDUwNjU5MTc5NywiaGVpZ2h0IjoxLjkwNTAwMDgwNTg1NDc5NzR9fSx7InBhZ2VOdW1iZXIiOjg5LCJiYm94Ijp7InRvcCI6NjUuOTU1Njg4NDc2NTYyNSwibGVmdCI6MTcuNSwid2lkdGgiOjYzLjA5NTY5OTMxMDMwMjczNCwiaGVpZ2h0IjoxLjkwNTAwMDgwNTg1NDc5NzR9fSx7InBhZ2VOdW1iZXIiOjg5LCJiYm94Ijp7InRvcCI6NjcuNTgzODkyODIyMjY1NjIsImxlZnQiOjE0LjcyMjg1MzY2MDU4MzQ5Niwid2lkdGgiOjY4LjUyMjU5ODI2NjYwMTU2LCJoZWlnaHQiOjEuOTA0OTk2NjMzNTI5NjYzfX0seyJwYWdlTnVtYmVyIjo4OSwiYmJveCI6eyJ0b3AiOjY5LjIxMjA4OTUzODU3NDIyLCJsZWZ0IjoxNC43MjI4NTM2NjA1ODM0OTYsIndpZHRoIjo2NS40MTIyNzcyMjE2Nzk2OSwiaGVpZ2h0IjoxLjkwNTAwMDgwNTg1NDc5NzR9fSx7InBhZ2VOdW1iZXIiOjg5LCJiYm94Ijp7InRvcCI6NzAuODQwMjkzODg0Mjc3MzQsImxlZnQiOjE0LjcyMjg1MzY2MDU4MzQ5Niwid2lkdGgiOjY3Ljc3MDAxOTUzMTI1LCJoZWlnaHQiOjEuOTA1MDA1MDk3Mzg5MjIxMn19LHsicGFnZU51bWJlciI6ODksImJib3giOnsidG9wIjo3Mi40Njg0OTgyMjk5ODA0NywibGVmdCI6MTQuNzIyODUzNjYwNTgzNDk2LCJ3aWR0aCI6NjkuMzg2MzI5NjUwODc4OSwiaGVpZ2h0IjoxLjkwNTAwNTA5NzM4OTIyMTJ9fSx7InBhZ2VOdW1iZXIiOjg5LCJiYm94Ijp7InRvcCI6NzQuMDk2NzAyNTc1NjgzNiwibGVmdCI6MTQuNzIyODUzNjYwNTgzNDk2LCJ3aWR0aCI6MzAuMTM0ODgwMDY1OTE3OTcsImhlaWdodCI6MS45MDUwMDUwOTczODkyMjEyfX0seyJwYWdlTnVtYmVyIjo4OSwiYmJveCI6eyJ0b3AiOjc1LjcyNDkxNDU1MDc4MTI1LCJsZWZ0IjoxNy41LCJ3aWR0aCI6NjQuNDEwNzk3MTE5MTQwNjIsImhlaWdodCI6MS45MDUwMDUwOTczODkyMjEyfX0seyJwYWdlTnVtYmVyIjo4OSwiYmJveCI6eyJ0b3AiOjc3LjM1MzExMTI2NzA4OTg0LCJsZWZ0IjoxNC43MjI4NTM2NjA1ODM0OTYsIndpZHRoIjo2NC44NTU2OTAwMDI0NDE0LCJoZWlnaHQiOjEuOTA1MDA1MDk3Mzg5MjIxMn19LHsicGFnZU51bWJlciI6ODksImJib3giOnsidG9wIjo3OC45ODEzMTU2MTI3OTI5NywibGVmdCI6MTQuNzIyODUzNjYwNTgzNDk2LCJ3aWR0aCI6NjYuMDE5ODUxNjg0NTcwMzEsImhlaWdodCI6MS45MDUwMDUwOTczODkyMjEyfX0seyJwYWdlTnVtYmVyIjo4OSwiYmJveCI6eyJ0b3AiOjgwLjYwOTUyNzU4Nzg5MDYyLCJsZWZ0IjoxNC43MjI4NTM2NjA1ODM0OTYsIndpZHRoIjo2Ny43ODc2NDM0MzI2MTcxOSwiaGVpZ2h0IjoxLjkwNTAwNTA5NzM4OTIyMTJ9fSx7InBhZ2VOdW1iZXIiOjg5LCJiYm94Ijp7InRvcCI6ODIuMjM3NzI0MzA0MTk5MjIsImxlZnQiOjE0LjcyMjg1MzY2MDU4MzQ5Niwid2lkdGgiOjY5LjczOTY5MjY4Nzk4ODI4LCJoZWlnaHQiOjEuOTA1MDA1MDk3Mzg5MjIxMn19LHsicGFnZU51bWJlciI6ODksImJib3giOnsidG9wIjo4My44NjU5MzYyNzkyOTY4OCwibGVmdCI6MTQuNzIyODUzNjYwNTgzNDk2LCJ3aWR0aCI6NjguNDg3MTI5MjExNDI1NzgsImhlaWdodCI6MS45MDUwMDUwOTczODkyMjEyfX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiOTAifX0seyJvcmlnaW4iOjUsIndvcmsiOnsidHlwZSI6OCwic3VidHlwZSI6eyJvbmVvZktpbmQiOiJhcnRpY2xlVHlwZSIsImFydGljbGVUeXBlIjoxfSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQXBvb3J2YSBNLiIsImZhbWlseSI6IlNhbXBhdCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBbmRyZWEiLCJmYW1pbHkiOiJIaWNrcyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJHZXJhcmRvIEouIiwiZmFtaWx5IjoiUnVpeuKAkE1lcmNhZG8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiVsOtwq1jdG9yIE0uIiwiZmFtaWx5IjoiWmF2YWxhIn19XSwiaWRzIjp7Imlzc24iOlsiMDkyMS0zNDQ5IiwiMTg3OS0wNjU4Il0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDE2L2oucmVzY29ucmVjLjIwMjAuMTA1MTk5IiwibWFnIjoiMzA5NDYyNTEwNCIsInBtaWQiOiIzMzc0NjM2MSIsIm9wZW5hbGV4IjoiVzMwOTQ2MjUxMDQifSwidGl0bGUiOiJWYWx1aW5nIGVjb25vbWljIGltcGFjdCByZWR1Y3Rpb25zIG9mIG51dHJpZW50IHBvbGx1dGlvbiBmcm9tIGxpdmVzdG9jayB3YXN0ZSIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2oucmVzY29ucmVjLjIwMjAuMTA1MTk5IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3d3dy5uY2JpLm5sbS5uaWguZ292L3BtYy9hcnRpY2xlcy83OTcwNTA1IiwiaXNzdWVkRGF0ZSI6IjIwMjAtMTAtMTUiLCJhY2Nlc3NlZERhdGUiOiIyMDI2LTAxIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJSZXNvdXJjZXMgQ29uc2VydmF0aW9uIGFuZCBSZWN5Y2xpbmciLCJ2b2x1bWUiOjE2NCwiaW1wYWN0RmFjdG9yIjoiMTEuMDEifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjoxMDUxOTksImxhc3RQYWdlIjoxMDUxOTl9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRWxzZXZpZXIgQlYifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjM5LCJjaXRlZEJ5Q291bnQiOjU3fX0sInF1b3RlIjoiU2VjdGlvbiBUaXRsZTogVmFsdWluZyBlY29ub21pYyBpbXBhY3QgcmVkdWN0aW9ucyBvZiBudXRyaWVudCBwb2xsdXRpb24gZnJvbSBsaXZlc3RvY2sgd2FzdGUgPiAxLiBJbnRyb2R1Y3Rpb25cbkNvbnRlbnQ6XG5JbiB0aGUgU3RhdGUgb2YgRmxvcmlkYSwgSEFCcyBoYXZlIHJlc3VsdGVkIGluIGEgbW9udGhseSBsb3NzIG9mIDIuOCBhbmQgMy43IG1pbGxpb24gVVNEIGNvcnJlc3BvbmRpbmcgdG8gcmVzdGF1cmFudCBhbmQgbG9kZ2luZyByZXZlbnVlLCByZXNwZWN0aXZlbHkgKCBbTGFya2luIGFuZCBBZGFtcywgMjAwN10oKSApLiBGcmVxdWVudCBvY2N1cnJlbmNlIG9mIEhBQnMgYWxzbyBsb3dlcnMgcHJvcGVydHkgdmFsdWVzIG9mIGxha2Vmcm9udCBwcm9wZXJ0aWVzLiBUaGUgbG9zcyBpbiBwcm9wZXJ0eSB2YWx1ZXMgYXJlIHRoZSBsYXJnZXN0IGltcGFjdCBiZWFyZXJzIG9mIGV1dHJvcGhpY2F0aW9uIHdpdGggYW4gZXN0aW1hdGVkIGF2ZXJhZ2UgZWNvbm9taWMgbG9zcyBvZiAxLjYgYmlsbGlvbiBVU0QgYW5udWFsbHkgKCBbRG9kZHMgZXQgYWwuLCAyMDA5XSgpICkuIn1d. This revival not only enhances monitoring granularity but paves the way for nature-based solutions, where revived tanks serve as living laboratories for floating wetlands and riparian buffers. These interventions leverage phytoremediation to sequester excess nitrogen and phosphorus, directly addressing the internal loading mechanisms that fuel persistent blooms (Everard et al., 2017)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1hcmsiLCJmYW1pbHkiOiJFdmVyYXJkIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik8uIFAuIiwiZmFtaWx5IjoiU2hhcm1hIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlZpbm9kIEt1bWFyIiwiZmFtaWx5IjoiVmlzaHdha2FybWEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiRGhhcm1lbmRyYSIsImZhbWlseSI6IktoYW5kYWwifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiWW9nZXNoIiwiZmFtaWx5IjoiU2FodSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJSYWh1bCIsImZhbWlseSI6IkJoYXRuYWdhciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJKaXRlbmRyYSBLdW1hciIsImZhbWlseSI6IlNpbmdoIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlJpdGVzaCIsImZhbWlseSI6Ikt1bWFyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFzZ2hhciIsImZhbWlseSI6Ik5hd2FiIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFtaXQiLCJmYW1pbHkiOiJLdW1hciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJWaXZlayIsImZhbWlseSI6Ikt1bWFyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFuaWwiLCJmYW1pbHkiOiJLYXNoeWFwIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkRlZXAgTmFyYXlhbiIsImZhbWlseSI6IlBhbmRleSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBZHJpYW4gQy4iLCJmYW1pbHkiOiJQaW5kZXIifX1dLCJpZHMiOnsiaXNzbiI6WyIwMDQ4LTk2OTciLCIxODc5LTEwMjYiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMTYvai5zY2l0b3RlbnYuMjAxNy4wOC4zMDgiLCJtYWciOiIyNzUzNDM1NzkxIiwicG1pZCI6IjI4ODkyODY4Iiwib3BlbmFsZXgiOiJXMjc1MzQzNTc5MSIsIm9wZW5BY2Nlc3NJZCI6IjE3OTQ2NTQ1MzE3MTEyMDI0ODA3In0sInRpdGxlIjoiQXNzZXNzaW5nIHRoZSBmZWFzaWJpbGl0eSBvZiBpbnRlZ3JhdGluZyBlY29zeXN0ZW0tYmFzZWQgd2l0aCBlbmdpbmVlcmVkIHdhdGVyIHJlc291cmNlIGdvdmVybmFuY2UgYW5kIG1hbmFnZW1lbnQgZm9yIHdhdGVyIHNlY3VyaXR5IGluIHNlbWktYXJpZCBsYW5kc2NhcGVzOiBBIGNhc2Ugc3R1ZHkgaW4gdGhlIEJhbmFzIGNhdGNobWVudCwgUmFqYXN0aGFuLCBJbmRpYSIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2ouc2NpdG90ZW52LjIwMTcuMDguMzA4IiwicmVzb3VyY2VVcmwiOiJodHRwOi8vZXByaW50cy5ib3VybmVtb3V0aC5hYy51ay8yOTY4NS8xLzIwMTctMDgtMjQlMjByZXN1Ym1pdHRlZCUyMHBhcGVyJTIwU1RPVEVOLUQtMTctMDUzMzlSMi5wZGYiLCJpc3N1ZWREYXRlIjoiMjAxNy0wOS0wOCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMDciLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IlRoZSBTY2llbmNlIG9mIFRoZSBUb3RhbCBFbnZpcm9ubWVudCIsInZvbHVtZSI6NjEyLCJpbXBhY3RGYWN0b3IiOiIxMS45NCJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjEyNDksImxhc3RQYWdlIjoxMjY1fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkVsc2V2aWVyIEJWIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo2MSwiY2l0ZWRCeUNvdW50Ijo2OH19LCJxdW90ZSI6IlRoZXJlIGlzIGEgZ3Jvd2luZyBsaXRlcmF0dXJlIDY1MiBhc3NlcnRpbmcgdGhhdCB0cmFkaXRpb25hbCBrbm93bGVkZ2UsIGN1cnJlbnRseSBiZWluZyBsb3N0IHRocm91Z2ggdmlsbGFnZSA2NTMgYWJhbmRvbm1lbnQgYW5kIGNvbnZlcnNpb24gdG8gbWVjaGFuaXNlZCB0ZWNobmlxdWVzLCBjYW4gcGxheSBzaWduaWZpY2FudCByb2xlcyBpbiA2NTQgcmViYWxhbmNpbmcgd2F0ZXIgcmVzb3VyY2UgcmVjaGFyZ2Ugd2l0aCBkZW1hbmRzIG9uIHJlY2VkaW5nIGdyb3VuZHdhdGVyIGlmIDY1NSBhcHByb3ByaWF0ZWx5IHN1cHBvcnRlZCBieSByZWZvcm1lZCBwb2xpY2llcyBhbmQgaW52ZXN0bWVudCAoZm9yIGV4YW1wbGUgU2hhaCBhbmQgNjU2IFJhanUsIDIwMDI7IFBhbmRleSBldCBhbC4sIDIwMDM7IFJhdGhvcmUsIDIwMDU7IE5hcmFpbiBldCBhbC4sIDIwMDU7IEV2ZXJhcmQsIDIwMTUpLiA2NTcgV2F0ZXJzaGVkIG1hbmFnZW1lbnQgcHJvZ3JhbW1lcyBwcm9tb3RpbmcgdGhlIGRpc3RyaWJ1dGVkIHJlc3RvcmF0aW9uIG9mIHNtYWxsLTY1OCBzY2FsZSB3YXRlciBoYXJ2ZXN0aW5nIGhhdmUgcmVzdWx0ZWQgaW4gc2lnbmlmaWNhbnQgaW1wYWN0cyBvbiBjYXRjaG1lbnQgaHlkcm9sb2d5IDY1OSBhbmQgZG93bnN0cmVhbSB3YXRlciBhdmFpbGFiaWxpdHkgaW4gQW5kaHJhIFByYWRlc2ggYW5kIG90aGVyIHBhcnRzIG9mIEluZGlhIChGQU8sIDY2MCAyMDEyKS4gU2lnbmlmaWNhbnQgZ3JvdW5kd2F0ZXIgcmlzZXMgYXJlIHJlcG9ydGVkIHdoZXJlIGNvbW11bml0eS1iYXNlZCA2NjEgcGFydGljaXBhdG9yeSBtZXRob2RzIGhhdmUgYmVlbiBkZXZlbG9wZWQgYXQgYmVuY2htYXJrIHNpdGVzIGluIHNldmVyYWwgSW5kaWFuIDY2MiBzdGF0ZXMvcHJvdmluY2VzIGFtb25nc3QgYSB3aWRlIHJhbmdlIG9mIGV4cGVyaW1lbnRhbCB3YXRlcnNoZWRzIGFjcm9zcyBBc2lhIDY2MyAoV2FuaSBldCBhbC4sIDIwMDMsIDIwMDUsIDIwMDYgYW5kIDIwMDkpLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6NjksImJib3giOnsidG9wIjo2My44MTYwMzYyMjQzNjUyMzQsImxlZnQiOjEyLjA5ODM2NzY5MTA0MDAzOSwid2lkdGgiOjczLjk2MDU2MzY1OTY2Nzk3LCJoZWlnaHQiOjEuOTU4Mzg1MzQ4MzIwMDA3M319LHsicGFnZU51bWJlciI6NjksImJib3giOnsidG9wIjo2NC4zNjQ3ODQyNDA3MjI2NiwibGVmdCI6Ni4yMjg1ODMzMzU4NzY0NjUsIndpZHRoIjozLjI2MTQxNTcxOTk4NTk2MiwiaGVpZ2h0IjoxLjMxMTI5MzEyNTE1MjU4OH19LHsicGFnZU51bWJlciI6NjksImJib3giOnsidG9wIjo2NS42NDA0MzQyNjUxMzY3MiwibGVmdCI6Ni4yMjg1ODMzMzU4NzY0NjUsIndpZHRoIjo2OS42OTc2Mzk0NjUzMzIwMywiaGVpZ2h0IjoxLjk1ODM4NTM0ODMyMDAwNzN9fSx7InBhZ2VOdW1iZXIiOjY5LCJiYm94Ijp7InRvcCI6NjcuNDM2MzMyNzAyNjM2NzIsImxlZnQiOjYuMjI4NTgzMzM1ODc2NDY1LCJ3aWR0aCI6ODIuMTU0MTIxMzk4OTI1NzgsImhlaWdodCI6MS45NTgzODUzNDgzMjAwMDczfX0seyJwYWdlTnVtYmVyIjo2OSwiYmJveCI6eyJ0b3AiOjY5LjI0NjQ3NTIxOTcyNjU2LCJsZWZ0Ijo2LjIyODU4MzMzNTg3NjQ2NSwid2lkdGgiOjc2LjY2NTk4NTEwNzQyMTg4LCJoZWlnaHQiOjEuOTU4Mzg1MzQ4MzIwMDA3M319LHsicGFnZU51bWJlciI6NjksImJib3giOnsidG9wIjo3MS4wNDIzODEyODY2MjExLCJsZWZ0Ijo2LjIyODU4MzMzNTg3NjQ2NSwid2lkdGgiOjgxLjMyMDMyMDEyOTM5NDUzLCJoZWlnaHQiOjEuOTU4Mzg1MzQ4MzIwMDA3M319LHsicGFnZU51bWJlciI6NjksImJib3giOnsidG9wIjo3Mi44MzgyNzk3MjQxMjExLCJsZWZ0Ijo2LjIyODU4MzMzNTg3NjQ2NSwid2lkdGgiOjgxLjE2NTc1NjIyNTU4NTk0LCJoZWlnaHQiOjEuOTU4Mzg1MzQ4MzIwMDA3M319LHsicGFnZU51bWJlciI6NjksImJib3giOnsidG9wIjo3NS41ODkxMjY1ODY5MTQwNiwibGVmdCI6Ni4yMjg1ODMzMzU4NzY0NjUsIndpZHRoIjo4MC44NzM0NjY0OTE2OTkyMiwiaGVpZ2h0IjoxLjk1ODM4NTM0ODMyMDAwNzN9fSx7InBhZ2VOdW1iZXIiOjY5LCJiYm94Ijp7InRvcCI6NzcuMjI4MjMzMzM3NDAyMzQsImxlZnQiOjYuMjI4NTgzMzM1ODc2NDY1LCJ3aWR0aCI6ODAuMjQwMjExNDg2ODE2NCwiaGVpZ2h0IjoxLjk1ODM4NTM0ODMyMDAwNzN9fSx7InBhZ2VOdW1iZXIiOjY5LCJiYm94Ijp7InRvcCI6NzguODY3MzQ3NzE3Mjg1MTYsImxlZnQiOjYuMjI4NTgzMzM1ODc2NDY1LCJ3aWR0aCI6ODEuMTI0Nzc4NzQ3NTU4NiwiaGVpZ2h0IjoxLjk1ODM4NTM0ODMyMDAwNzN9fSx7InBhZ2VOdW1iZXIiOjY5LCJiYm94Ijp7InRvcCI6ODAuNTA2NDU0NDY3NzczNDQsImxlZnQiOjYuMjI4NTgzMzM1ODc2NDY1LCJ3aWR0aCI6NzMuMjE1NzUxNjQ3OTQ5MjIsImhlaWdodCI6MS45NTgzODUzNDgzMjAwMDczfX0seyJwYWdlTnVtYmVyIjo2OSwiYmJveCI6eyJ0b3AiOjgyLjE0NTU2ODg0NzY1NjI1LCJsZWZ0Ijo2LjIyODU4MzMzNTg3NjQ2NSwid2lkdGgiOjc4LjIwODAyMzA3MTI4OTA2LCJoZWlnaHQiOjEuOTU4Mzg1MzQ4MzIwMDA3M319LHsicGFnZU51bWJlciI6NjksImJib3giOnsidG9wIjo4My43ODcwNjM1OTg2MzI4MSwibGVmdCI6Ni4yMjg1ODMzMzU4NzY0NjUsIndpZHRoIjo3Ny41MzY3NzM2ODE2NDA2MiwiaGVpZ2h0IjoxLjk1ODM4NTM0ODMyMDAwNzN9fSx7InBhZ2VOdW1iZXIiOjY5LCJiYm94Ijp7InRvcCI6ODUuNDExOTExMDEwNzQyMTksImxlZnQiOjEyLjA5ODM2NzY5MTA0MDAzOSwid2lkdGgiOjcyLjU4NTQ5NDk5NTExNzE5LCJoZWlnaHQiOjEuOTU4Mzg1MzQ4MzIwMDA3M319XX0sImNodW5rSWQiOiJlZWQ4ZTBmOS1hNWVjLTQ1NWQtYWY3OC00ZTVmMjE3NjI2NTgiLCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIxMzE4In19XQ%3D%3D. These interventions leverage phytoremediation to sequester excess nitrogen and phosphorus, directly addressing the internal loading mechanisms that fuel persistent blooms, while simultaneously enhancing habitat complexity and aesthetic value (Jain et al., 2024; Singh, 2025)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNodWJoYW0iLCJmYW1pbHkiOiJKYWluIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFtYW4iLCJmYW1pbHkiOiJTcml2YXN0YXZhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkRpbmVzaCBLdW1hciIsImZhbWlseSI6IlZpc2h3YWthcm1hIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkppdGVuZHJhIiwiZmFtaWx5IjoiUmFqcHV0In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5pdGluIExpbGFkaGFyIiwiZmFtaWx5IjoiUmFuZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBbGkgQXNnaGFyIiwiZmFtaWx5IjoiU2FsZW0ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQWhtZWQiLCJmYW1pbHkiOiJFbGJlbHRhZ2kifX1dLCJpZHMiOnsiaXNzbiI6WyIyNjI0LTkzNzUiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzODkvZnJ3YS4yMDI0LjE0NDEzNjUiLCJvcGVuYWxleCI6Ilc0NDAxOTI5NTc2In0sInRpdGxlIjoiUHJvdGVjdGluZyBhbmNpZW50IHdhdGVyIGhhcnZlc3RpbmcgdGVjaG5vbG9naWVzIGluIEluZGlhOiBzdHJhdGVnaWVzIGZvciBjbGltYXRlIGFkYXB0YXRpb24gYW5kIHN1c3RhaW5hYmxlIGRldmVsb3BtZW50IHdpdGggZ2xvYmFsIGxlc3NvbnMiLCJhYnN0cmFjdCI6IkludHJvZHVjdGlvbiBBbmNpZW50IHdhdGVyIGhhcnZlc3Rpbmcgc3lzdGVtcywgc3VjaCBhcyB0aG9zZSBmcm9tIHRoZSBJbmR1cyBWYWxsZXkgQ2l2aWxpemF0aW9uICh%2BMzUwMCBCQ0UpLCBoYXZlIGJlZW4gdml0YWwgZm9yIGlycmlnYXRpb24gYW5kIGNsaW1hdGUgcmVzaWxpZW5jZSwgZXNwZWNpYWxseSBpbiBhcmlkIHJlZ2lvbnMuIE9uZSBzdWNoIHByb21pbmVudCBzeXN0ZW0gaW4gU291dGggQXNpYSwgY2FsbGVkIHRhbmsgaXJyaWdhdGlvbiwgaW5pdGlhbGx5IHRocml2ZWQgdGhyb3VnaCBjb21tdW5pdHkgbWFuYWdlbWVudCBidXQgZGVjbGluZWQgcG9zdC1pbmRlcGVuZGVuY2UgZHVlIHRvIGNvbG9uaWFsIHBvbGljaWVzIGFuZCBuZWdsZWN0IGluIFNyaSBMYW5rYSBhbmQgSW5kaWEuIFRoaXMgc3R1ZHkgZXZhbHVhdGVzIGN1cnJlbnQgcG9saWN5IGZyYW1ld29ya3MgYW5kIHJlaGFiaWxpdGF0aW9uIHByb2dyYW1zIHRvIGVuaGFuY2UgdGhlIHJlc2lsaWVuY2Ugb2YgdGhlc2Ugc3lzdGVtcyBpbiBJbmRpYSwgZGV2ZWxvcCBzdHJhdGVnaWVzIGZvciB0aGVpciBwcm90ZWN0aW9uIGFuZCBhZGFwdGF0aW9uIHRvIGNsaW1hdGUgY2hhbmdlLCBhbmQgaW50ZWdyYXRlIGdsb2JhbCBsZXNzb25zIGZvciBzdXN0YWluYWJsZSBkZXZlbG9wbWVudC4gTWV0aG9kcyBBIHN5c3RlbWF0aWMgbWV0YS1hbmFseXNpcyBvZiBncmV5IGxpdGVyYXR1cmUgd2FzIGNvbmR1Y3RlZCB0byBhZ2dyZWdhdGUgZGF0YSBvbiBwb2xpY3kgY29uc3RyYWludHMuIFBvbGljeSBhbmFseXNpcyBpbnZvbHZlZCBkZXRhaWxlZCBpbnZlc3RpZ2F0aW9ucyBvZiByZWxldmFudCBkb2N1bWVudHMsIHJlZ3VsYXRpb25zLCBhbmQgY29tcGFyYXRpdmUgYW5hbHlzZXMgb2YgZnJhbWV3b3JrcyBhdCByZWdpb25hbCBhbmQgbmF0aW9uYWwgbGV2ZWxzLiBQaWxvdCBwcm9qZWN0cyBvbiB0YW5rIHJlaGFiaWxpdGF0aW9uIHdlcmUgYXNzZXNzZWQgdGhyb3VnaCByZXBvcnRlZCBjYXNlIHN0dWRpZXMgYW5kIGZpZWxkIHN1cnZleXMgdG8gZ2F1Z2UgaW1wYWN0LiBUaGVtYXRpYyBhbmFseXNpcyB3YXMgdXNlZCB0byBleHBsb3JlIHRoZSBnbG9iYWwgcG90ZW50aWFsIG9mIHRoZXNlIHN5c3RlbXMgaW4gY2xpbWF0ZSByZXNpbGllbmNlIGFuZCBvdmVyYWxsIGVudmlyb25tZW50YWwgc3VzdGFpbmFiaWxpdHkuIFJlc3VsdHMgVGhlIGFuYWx5c2lzIHNob3dlZCB0aGF0IHBpbG90IHByb2plY3RzIGZvciB0YW5rIHJlaGFiaWxpdGF0aW9uIGhhZCBsaW1pdGVkIHN1Y2Nlc3MgaW4gYWNoaWV2aW5nIHN1c3RhaW5hYmlsaXR5IHVuZGVyIGN1cnJlbnQgY2xpbWF0ZSBjb25kaXRpb25zLiBUYW5rIGlycmlnYXRpb24gc3lzdGVtcyBhcmUgY3J1Y2lhbCBmb3IgYWRhcHRpbmcgdG8gZXh0cmVtZSB3ZWF0aGVyLCBpbmNsdWRpbmcgZmxvb2RzLCBkcm91Z2h0cywgYW5kIGhlYXQgd2F2ZXMsIHJlcGxlbmlzaGluZyBncm91bmR3YXRlciwgcmVkdWNpbmcgc29pbCBlcm9zaW9uLCBhbmQgZW5zdXJpbmcgcmVsaWFibGUgd2F0ZXIgc3VwcGxpZXMuIFRyYWRpdGlvbmFsIHdhdGVyIGhhcnZlc3RpbmcgdGVjaG5vbG9naWVzIHN1cHBvcnQgMTcgU3VzdGFpbmFibGUgRGV2ZWxvcG1lbnQgR29hbHMgKFNER3MpLCBpbmNsdWRpbmcgY2xlYW4gd2F0ZXIgYWNjZXNzLCBodW5nZXIgcmVkdWN0aW9uLCBnZW5kZXIgZXF1YWxpdHksIGFuZCBjbGltYXRlIGFjdGlvbi4gSW50ZWdyYXRpbmcgQUkgYW5kIG1hY2hpbmUgbGVhcm5pbmcgaW4gd2F0ZXIgbWFuYWdlbWVudCBiZW5lZml0cyBkaXNhc3RlciByZXNwb25zZSwgd2hpbGUgZWNvLXRvdXJpc20gYWlkcyBzeXN0ZW0gbWFpbnRlbmFuY2UgYW5kIGN1bHR1cmFsIGF3YXJlbmVzcy4gRGlzY3Vzc2lvbiBUaGUgc3R1ZHkgdW5kZXJzY29yZXMgdGhlIG5lZWQgZm9yIHBvbGljeSByZWZvcm1zIHRvIGVuaGFuY2UgdGFuayByZWhhYmlsaXRhdGlvbiBhbmQgaW5zdGl0dXRpb25hbCBhcnJhbmdlbWVudHMuIEl0IGNhbGxzIGZvciBpbmNyZWFzZWQgYmVuZWZpY2lhcnkgcGFydGljaXBhdGlvbiBhbmQgY29uc3RpdHV0aW9uYWwgcmVjb2duaXRpb24gb2YgY3VycmVudCBwcmFjdGljZXMuIFN0cmF0ZWdpYywgbmF0aW9uYWwtc2NhbGUgYXNzZXNzbWVudHMgYW5kIHJlc2lsaWVuY2UgdGFyZ2V0cyBhcmUgcmVjb21tZW5kZWQgdG8gaW1wcm92ZSB0aGUgZWZmZWN0aXZlbmVzcyBvZiBzdWNoIHdhdGVyIGhhcnZlc3Rpbmcgc3lzdGVtcyBpbiBtaXRpZ2F0aW5nIG5hdHVyYWwgaGF6YXJkcyBhbmQgZW5oYW5jaW5nIGVudmlyb25tZW50YWwgc2VydmljZXMuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjMzODkvZnJ3YS4yMDI0LjE0NDEzNjUiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzg5L2Zyd2EuMjAyNC4xNDQxMzY1IiwiaXNzdWVkRGF0ZSI6IjIwMjQtMDgtMjciLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTExIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJGcm9udGllcnMgaW4gV2F0ZXIiLCJ2b2x1bWUiOjYsImltcGFjdEZhY3RvciI6IjMuOTMifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRnJvbnRpZXJzIE1lZGlhIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo4OCwiY2l0ZWRCeUNvdW50IjoxNn19LCJxdW90ZSI6IlNlY3Rpb24gVGl0bGU6IFByb3RlY3RpbmcgYW5jaWVudCB3YXRlciBoYXJ2ZXN0aW5nIHRlY2hub2xvZ2llcyBpbiBJbmRpYTogc3RyYXRlZ2llcyBmb3IgY2xpbWF0ZSBhZGFwdGF0aW9uIGFuZCAuLi5cbkNvbnRlbnQ6XG4qKkludHJvZHVjdGlvbjoqKiBBbmNpZW50IHdhdGVyIGhhcnZlc3Rpbmcgc3lzdGVtcywgc3VjaCBhcyB0aG9zZSBmcm9tIHRoZSBJbmR1cyBWYWxsZXkgQ2l2aWxpemF0aW9uICh%2BMzUwMCBCQ0UpLCBoYXZlIGJlZW4gdml0YWwgZm9yIGlycmlnYXRpb24gYW5kIGNsaW1hdGUgcmVzaWxpZW5jZSwgZXNwZWNpYWxseSBpbiBhcmlkIHJlZ2lvbnMuIE9uZSBzdWNoIHByb21pbmVudCBzeXN0ZW0gaW4gU291dGggQXNpYSwgY2FsbGVkIHRhbmsgaXJyaWdhdGlvbiwgaW5pdGlhbGx5IHRocml2ZWQgdGhyb3VnaCBjb21tdW5pdHkgbWFuYWdlbWVudCBidXQgZGVjbGluZWQgcG9zdC1pbmRlcGVuZGVuY2UgZHVlIHRvIGNvbG9uaWFsIHBvbGljaWVzIGFuZCBuZWdsZWN0IGluIFNyaSBMYW5rYSBhbmQgSW5kaWEuIFRoaXMgc3R1ZHkgZXZhbHVhdGVzIGN1cnJlbnQgcG9saWN5IGZyYW1ld29ya3MgYW5kIHJlaGFiaWxpdGF0aW9uIHByb2dyYW1zIHRvIGVuaGFuY2UgdGhlIHJlc2lsaWVuY2Ugb2YgdGhlc2Ugc3lzdGVtcyBpbiBJbmRpYSwgZGV2ZWxvcCBzdHJhdGVnaWVzIGZvciB0aGVpciBwcm90ZWN0aW9uIGFuZCBhZGFwdGF0aW9uIHRvIGNsaW1hdGUgY2hhbmdlLCBhbmQgaW50ZWdyYXRlIGdsb2JhbCBsZXNzb25zIGZvciBzdXN0YWluYWJsZSBkZXZlbG9wbWVudC4iLCJleHRlcm5hbENhY2hlSWQiOiI5M2ViZWM1OC0yYWMwLTMwOGItOGQ1OS1kYTM2OTQwYmY3MDgifSx7Im9yaWdpbiI6NCwid29yayI6eyJ0eXBlIjo4LCJzdWJ0eXBlIjp7Im9uZW9mS2luZCI6ImFydGljbGVUeXBlIiwiYXJ0aWNsZVR5cGUiOjF9LCJjb250cmlidXRvcnMiOlt7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTdXJhaiBLdW1hciIsImZhbWlseSI6IlNpbmdoIn19XSwiaWRzIjp7Imlzc24iOlsiMjMyMS05NjUzIl0sImlzYm4iOltdLCJkb2kiOiIxMC4yMjIxNC9panJhc2V0LjIwMjUuNzMxMjgiLCJvcGVuYWxleCI6Ilc0NDEzMDgxMzAwIiwib3BlbkFjY2Vzc0lkIjoiMTQzMjQ1NzU1NjExOTcyODcxNTUifSwidGl0bGUiOiJSYWlud2F0ZXIgSGFydmVzdGluZyBTeXN0ZW1zIGluIFVyYmFuIGFuZCBSdXJhbCBJbmRpYTogUGVyZm9ybWFuY2UgYW5kIFBvbGljeSBSZXZpZXciLCJhYnN0cmFjdCI6IkNvbnRlbXBvcmFyeSBJbmRpYSBmYWNlcyB1bnByZWNlZGVudGVkIHdhdGVyIGNoYWxsZW5nZXMgYXMgZGVtb2dyYXBoaWMgcHJlc3N1cmVzIGludGVuc2lmeSBhbmQgY2xpbWF0aWMgdmFyaWFiaWxpdHkgZGlzcnVwdHMgZXN0YWJsaXNoZWQgcHJlY2lwaXRhdGlvbiBwYXR0ZXJucy4gVGhpcyBzY2VuYXJpbyBoYXMgcmVraW5kbGVkIGludGVyZXN0IGluIHJhaW53YXRlciBoYXJ2ZXN0aW5nIChSV0gpIG1ldGhvZG9sb2dpZXMgdGhhdCBoaXN0b3JpY2FsbHkgc3VzdGFpbmVkIEluZGlhbiBjb21tdW5pdGllcyBhY3Jvc3MgZGl2ZXJzZSBlY29sb2dpY2FsIHpvbmVzLiBPdXIgaW52ZXN0aWdhdGlvbiBleGFtaW5lcyB0aGUgb3BlcmF0aW9uYWwgZWZmZWN0aXZlbmVzcyBvZiBSV0ggaW1wbGVtZW50YXRpb25zIHNwYW5uaW5nIG1ldHJvcG9saXRhbiBhcmVhcyBhbmQgcmVtb3RlIHNldHRsZW1lbnRzLiBUaHJvdWdoIGNvbXByZWhlbnNpdmUgYW5hbHlzaXMgb2YgaW5kaWdlbm91cyB3YXRlciBjb25zZXJ2YXRpb24gcHJhY3RpY2VzIGluY2x1ZGluZyBCdW5kZWxraGFuZCdzIEhhdmVsaSBzdHJ1Y3R1cmVzIGFuZCBSYWphc3RoYW4ncyBDaGF1a2EgbmV0d29ya3MsIGFsb25nc2lkZSBjb250ZW1wb3Jhcnkgcm9vZnRvcCBjb2xsZWN0aW9uIHRlY2hub2xvZ2llcywgdGhpcyBzdHVkeSBwcmVzZW50cyBhIG51YW5jZWQgYXNzZXNzbWVudCBvZiBjdXJyZW50IHBlcmZvcm1hbmNlIG1ldHJpY3MuIFRoZSByZXNlYXJjaCBzeW50aGVzaXplcyBmaW5kaW5ncyBmcm9tIG11bHRpcGxlIGNhc2Ugc3R1ZGllcyB0byBldmFsdWF0ZSBzeXN0ZW0gZWZmaWNpZW5jeSwgZWNvbm9taWMgZmVhc2liaWxpdHksIGFuZCBwb2xpY3kgaW50ZWdyYXRpb24gY2hhbGxlbmdlcy4gUmVzdWx0cyBkZW1vbnN0cmF0ZSBzaWduaWZpY2FudCBwb3RlbnRpYWwgZm9yIGVuaGFuY2luZyByZWdpb25hbCB3YXRlciBzZWN1cml0eSB0aHJvdWdoIHN0cmF0ZWdpYyBSV0ggZGVwbG95bWVudCwgd2hpbGUgc2ltdWx0YW5lb3VzbHkgcmV2ZWFsaW5nIHBlcnNpc3RlbnQgb2JzdGFjbGVzIGluIHN5c3RlbSBtYWludGVuYW5jZSBwcm90b2NvbHMsIHdhdGVyIHF1YWxpdHkgbWFuYWdlbWVudCwgYW5kIGluc3RpdHV0aW9uYWwgY29vcmRpbmF0aW9uLiBUaGUgYW5hbHlzaXMgdW5kZXJzY29yZXMgdGhlIG5lY2Vzc2l0eSBmb3IgcmVmaW5lZCBwb2xpY3kgZnJhbWV3b3JrcyB0aGF0IGJyaWRnZSB0cmFkaXRpb25hbCBlY29sb2dpY2FsIGtub3dsZWRnZSB3aXRoIG1vZGVybiBlbmdpbmVlcmluZyBhcHByb2FjaGVzIHRvIGFjaGlldmUgc3VzdGFpbmFibGUgd2F0ZXIgbWFuYWdlbWVudCBvdXRjb21lcy4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjIyMTQvaWpyYXNldC4yMDI1LjczMTI4IiwicmVzb3VyY2VVcmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjIyMTQvaWpyYXNldC4yMDI1LjczMTI4IiwiaXNzdWVkRGF0ZSI6IjIwMjUtMDctMTciLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA5IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJJbnRlcm5hdGlvbmFsIEpvdXJuYWwgZm9yIFJlc2VhcmNoIGluIEFwcGxpZWQgU2NpZW5jZSBhbmQgRW5naW5lZXJpbmcgVGVjaG5vbG9neSIsImlzc3VlIjo3LCJ2b2x1bWUiOjEzLCJpbXBhY3RGYWN0b3IiOiIwLjQxIn0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTE2NSwibGFzdFBhZ2UiOjExNzF9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiSW50ZXJuYXRpb25hbCBKb3VybmFsIGZvciBSZXNlYXJjaCBpbiBBcHBsaWVkIFNjaWVuY2UgYW5kIEVuZ2luZWVyaW5nIFRlY2hub2xvZ3kgKElKUkFTRVQpIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjowLCJjaXRlZEJ5Q291bnQiOjB9fSwicXVvdGUiOiJIaXN0b3JpY2FsIGFuYWx5c2lzIHJldmVhbHMgc29waGlzdGljYXRlZCBpbmRpZ2Vub3VzIHdhdGVyIGNvbnNlcnZhdGlvbiBtZXRob2RvbG9naWVzIHRoYXQgZW5hYmxlZCAgc3VzdGFpbmFibGUgcmVzb3VyY2UgbWFuYWdlbWVudCBhY3Jvc3MgZGl2ZXJzZSBlY29sb2dpY2FsIHpvbmVzLiBBcmNoYWVvbG9naWNhbCBldmlkZW5jZSBmcm9tIEJ1bmRlbGtoYW5kJ3MgSGF2ZWxpIG5ldHdvcmtzICBhbmQgUmFqYXN0aGFuJ3MgdHJhZGl0aW9uYWwgcmVjaGFyZ2Ugc3lzdGVtcyBkZW1vbnN0cmF0ZXMgcmVtYXJrYWJsZSBlbmdpbmVlcmluZyBpbmdlbnVpdHkgYWRhcHRlZCB0byBsb2NhbCBlbnZpcm9ubWVudGFsICBjb25kaXRpb25zLiBUaGVzZSBwcmFjdGljZXMgc3VzdGFpbmVkIGFncmljdWx0dXJhbCBwcm9kdWN0aXZpdHkgYW5kIGNvbW11bml0eSByZXNpbGllbmNlIHRocm91Z2ggbXVsdGlwbGUgY2xpbWF0aWMgY3ljbGVzIGJlZm9yZSAgYmVpbmcgZGlzcGxhY2VkIGJ5IGNlbnRyYWxpemVkIGluZnJhc3RydWN0dXJlIHBhcmFkaWdtcy4gIENvbnRlbXBvcmFyeSByZWFzc2Vzc21lbnQgb2YgZGVjZW50cmFsaXplZCB3YXRlciBtYW5hZ2VtZW50IGFwcHJvYWNoZXMgcmVmbGVjdHMgZ3Jvd2luZyByZWNvZ25pdGlvbiBvZiBpbmZyYXN0cnVjdHVyZSAgdnVsbmVyYWJpbGl0eSBhbmQgcmVzb3VyY2Ugc2VjdXJpdHkgaW1wZXJhdGl2ZXMuIERlY2xpbmluZyBncm91bmR3YXRlciB0YWJsZXMsIGluY3JlYXNlZCBmbG9vZCBmcmVxdWVuY3kgaW4gdXJiYW4gYXJlYXMsIGFuZCAgaGVpZ2h0ZW5lZCBkcm91Z2h0IHZ1bG5lcmFiaWxpdHkgaW4gcnVyYWwgcmVnaW9ucyBuZWNlc3NpdGF0ZSBpbnRlZ3JhdGVkIHNvbHV0aW9ucyB0aGF0IGNvbWJpbmUgdHJhZGl0aW9uYWwgZWNvbG9naWNhbCBrbm93bGVkZ2Ugd2l0aCAgbW9kZXJuIHRlY2hub2xvZ2ljYWwgY2FwYWJpbGl0aWVzLiAgVGhpcyBpbnZlc3RpZ2F0aW9uIGFkb3B0cyBhIG11bHRpZGlzY2lwbGluYXJ5IGFwcHJvYWNoIHRvIGV2YWx1YXRlIFJXSCBzeXN0ZW0gcGVyZm9ybWFuY2UgYWNyb3NzIHZhcmllZCBpbXBsZW1lbnRhdGlvbiBjb250ZXh0cy4gIFJhdGhlciB0aGFuIGZvY3VzaW5nIGV4Y2x1c2l2ZWx5IG9uIHRlY2huaWNhbCBlZmZpY2llbmN5IG1ldHJpY3MsIHRoZSBhbmFseXNpcyBpbmNvcnBvcmF0ZXMgc29jaW8tZWNvbm9taWMgZmFjdG9ycywgaW5zdGl0dXRpb25hbCAgY2FwYWNpdHkgY29uc3RyYWludHMsIGFuZCBwb2xpY3kgaW1wbGVtZW50YXRpb24gY2hhbGxlbmdlcyB0aGF0IGRldGVybWluZSBsb25nLXRlcm0gc3VzdGFpbmFiaWxpdHkgb3V0Y29tZXMuIFRoZSByZXNlYXJjaCAgZnJhbWV3b3JrIGVtcGhhc2l6ZXMgcHJhY3RpY2FsIGltcGxlbWVudGF0aW9uIGluc2lnaHRzIGRlcml2ZWQgZnJvbSBmaWVsZCBvYnNlcnZhdGlvbnMgYW5kIHN0YWtlaG9sZGVyIGVuZ2FnZW1lbnQgYWNyb3NzICBtdWx0aXBsZSBnZW9ncmFwaGljYWwgY29udGV4dHMgd2l0aGluIEthcm5hdGFrYSBhbmQgYWRqYWNlbnQgc3RhdGVzLiBJSS4gTUVUSE9ET0xPR1kgVGhpcyBpbnZlc3RpZ2F0aW9uIGVtcGxveXMgc3lzdGVtYXRpYyBsaXRlcmF0dXJlIHN5bnRoZXNpcyBjb21iaW5lZCB3aXRoIHRoZW1hdGljIGFuYWx5c2lzIHRvIGV2YWx1YXRlIHJhaW53YXRlciBoYXJ2ZXN0aW5nIGltcGxlbWVudGF0aW9ucyBhY3Jvc3MgZGl2ZXJzZSBJbmRpYW4gY29udGV4dHMuIFRoZSBtZXRob2RvbG9naWNhbCBhcHByb2FjaCBwcmlvcml0aXplcyBlbXBpcmljYWwgZXZpZGVuY2UgZnJvbSBwZWVyLXJldmlld2VkICBwdWJsaWNhdGlvbnMgc3Bhbm5pbmcgdGhlIGRlY2FkZSAyMDE0LTIwMjMsIGVtcGhhc2l6aW5nIHN0dWRpZXMgdGhhdCBwcm92aWRlIHF1YW50aXRhdGl2ZSBwZXJmb3JtYW5jZSBkYXRhIGFuZCBxdWFsaXRhdGl2ZSAgaW1wbGVtZW50YXRpb24gaW5zaWdodHMuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTQuODExMzMyNzAyNjM2NzIsImxlZnQiOjYuNTg4MjM1Mzc4MjY1MzgxLCJ3aWR0aCI6ODcuMjI3MzU1OTU3MDMxMjUsImhlaWdodCI6MS42OTc4MTc2ODMyMTk5MDk3fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTYuNDc4MDA0NDU1NTY2NDA2LCJsZWZ0Ijo2LjU4ODIzNTM3ODI2NTM4MSwid2lkdGgiOjg3LjIyOTMwMTQ1MjYzNjcyLCJoZWlnaHQiOjEuNjk3ODE3NjgzMjE5OTA5N319LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjU4LjE0NDY2NDc2NDQwNDMsImxlZnQiOjYuNTg4MjM1Mzc4MjY1MzgxLCJ3aWR0aCI6ODcuMjIzNDI2ODE4ODQ3NjYsImhlaWdodCI6MS42OTc4MTc2ODMyMTk5MDk3fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTkuODExMzM2NTE3MzMzOTg0LCJsZWZ0Ijo2LjU4ODIzNTM3ODI2NTM4MSwid2lkdGgiOjg3LjIzNTY1NjczODI4MTI1LCJoZWlnaHQiOjEuNjk3ODE3NjgzMjE5OTA5N319LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjYxLjQ3ODAwMDY0MDg2OTE0LCJsZWZ0Ijo2LjU4ODIzNTM3ODI2NTM4MSwid2lkdGgiOjM3LjA3NzY0NDM0ODE0NDUzLCJoZWlnaHQiOjEuNjk3ODE3NjgzMjE5OTA5N319LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjYzLjE0NDY2NDc2NDQwNDMsImxlZnQiOjYuNTg4MjM1Mzc4MjY1MzgxLCJ3aWR0aCI6ODcuMjM1MTQ1NTY4ODQ3NjYsImhlaWdodCI6MS42OTc4MTc2ODMyMTk5MDk3fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NjQuODExMzQwMzMyMDMxMjUsImxlZnQiOjYuNTg4MjM1Mzc4MjY1MzgxLCJ3aWR0aCI6ODcuMjM1OTkyNDMxNjQwNjIsImhlaWdodCI6MS42OTc4MTc2ODMyMTk5MDk3fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NjYuNDc3OTk2ODI2MTcxODgsImxlZnQiOjYuNTg4MjM1Mzc4MjY1MzgxLCJ3aWR0aCI6ODcuMjI5ODg4OTE2MDE1NjIsImhlaWdodCI6MS42OTc4MTc2ODMyMTk5MDk3fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NjguMTQ0NjY4NTc5MTAxNTYsImxlZnQiOjYuNTg4MjM1Mzc4MjY1MzgxLCJ3aWR0aCI6MjIuODgxNTcyNzIzMzg4NjcyLCJoZWlnaHQiOjEuNjk3ODE3NjgzMjE5OTA5N319LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjY5Ljg0MTYzNjY1NzcxNDg0LCJsZWZ0Ijo2LjU4ODIzNTM3ODI2NTM4MSwid2lkdGgiOjg3LjIzNzcyNDMwNDE5OTIyLCJoZWlnaHQiOjEuNjk3ODE3NjgzMjE5OTA5N319LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjcxLjUwODMwODQxMDY0NDUzLCJsZWZ0Ijo2LjU4ODIzNTM3ODI2NTM4MSwid2lkdGgiOjg3LjIzNjkzODQ3NjU2MjUsImhlaWdodCI6MS42OTc4MTc2ODMyMTk5MDk3fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NzMuMTc0OTcyNTM0MTc5NjksImxlZnQiOjYuNTg4MjM1Mzc4MjY1MzgxLCJ3aWR0aCI6ODcuMjM1MDkyMTYzMDg1OTQsImhlaWdodCI6MS42OTc4MTc2ODMyMTk5MDk3fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NzQuODQxNjI5MDI4MzIwMzEsImxlZnQiOjYuNTg4MjM1Mzc4MjY1MzgxLCJ3aWR0aCI6ODcuMjI5ODQzMTM5NjQ4NDQsImhlaWdodCI6MS42OTc4MTc2ODMyMTk5MDk3fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NzYuNTA4MzAwNzgxMjUsImxlZnQiOjYuNTg4MjM1Mzc4MjY1MzgxLCJ3aWR0aCI6NDUuMzEyOTM4NjkwMTg1NTUsImhlaWdodCI6MS42OTc4MTc2ODMyMTk5MDk3fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NzkuODUwMzAzNjQ5OTAyMzQsImxlZnQiOjQxLjYwNzg0MTQ5MTY5OTIyLCJ3aWR0aCI6MTcuMTk1MjkxNTE5MTY1MDQsImhlaWdodCI6MS43OTU4MTMwODM2NDg2ODE2fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6ODEuNTA4MzAwNzgxMjUsImxlZnQiOjYuNTg4MjM1Mzc4MjY1MzgxLCJ3aWR0aCI6ODcuMjMzMzY3OTE5OTIxODgsImhlaWdodCI6MS42OTc4MTc2ODMyMTk5MDk3fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6ODMuMTc0OTY0OTA0Nzg1MTYsImxlZnQiOjYuNTg4MjM1Mzc4MjY1MzgxLCJ3aWR0aCI6ODcuMjM2MjU5NDYwNDQ5MjIsImhlaWdodCI6MS42OTc4MTc2ODMyMTk5MDk3fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6ODQuODQxNjI5MDI4MzIwMzEsImxlZnQiOjYuNTg4MjM1Mzc4MjY1MzgxLCJ3aWR0aCI6ODcuMjMwMDcyMDIxNDg0MzgsImhlaWdodCI6MS42OTc4MTc2ODMyMTk5MDk3fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6ODYuNTM4NjA0NzM2MzI4MTIsImxlZnQiOjYuNTg4MjM1Mzc4MjY1MzgxLCJ3aWR0aCI6ODcuMjMyMzE1MDYzNDc2NTYsImhlaWdodCI6MS42OTc4MTc2ODMyMTk5MDk3fX1dfSwiY2h1bmtJZCI6IjllODQ4ZWQyLWJhYWItNDE2OC1hNjhiLTVkYTg1Y2I2ZmU3OCIsImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjExNjYifX1d.

14. Decentralized, Nature-Based Solutions for Community Action

Decentralized, nature-based solutions empower local communities to combat eutrophication and harmful algal blooms in Indian lakes through low-cost, scalable interventions that mimic natural processes. Unlike centralized infrastructure, these approaches such as floating wetlands, riparian buffer zones, and localized waste management leverage ecosystem services to sequester nutrients, enhance biodiversity, and build resilience against climate stressors like warming-induced stratification (Paerl et al., 2019)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhhbnMgVy4iLCJmYW1pbHkiOiJQYWVybCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLYXJsIEUuIiwiZmFtaWx5IjoiSGF2ZW5zIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hdGhhbiBTLiIsImZhbWlseSI6IkhhbGwifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiVGltb3RoeSBHLiIsImZhbWlseSI6Ik90dGVuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1lbmd5dWFuIiwiZmFtaWx5IjoiWmh1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhhaSIsImZhbWlseSI6Ilh1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikd1YW5nd2VpIiwiZmFtaWx5IjoiWmh1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkJvcWlhbmciLCJmYW1pbHkiOiJRaW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIxMzIzLTE2NTAiLCIxNDQ4LTYwNTkiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwNzEvbWYxODM5MiIsIm1hZyI6IjI5NDIzMjE1NDIiLCJvcGVuYWxleCI6IlcyOTQyMzIxNTQyIn0sInRpdGxlIjoiTWl0aWdhdGluZyBhIGdsb2JhbCBleHBhbnNpb24gb2YgdG94aWMgY3lhbm9iYWN0ZXJpYWwgYmxvb21zOiBjb25mb3VuZGluZyBlZmZlY3RzIGFuZCBjaGFsbGVuZ2VzIHBvc2VkIGJ5IGNsaW1hdGUgY2hhbmdlIiwiYWJzdHJhY3QiOiJNYW5hZ2luZyBhbmQgbWl0aWdhdGluZyB0aGUgZ2xvYmFsIGV4cGFuc2lvbiBvZiB0b3hpYyBjeWFub2JhY3RlcmlhbCBoYXJtZnVsIGFsZ2FsIGJsb29tcyAoQ3lhbm9IQUJzKSBpcyBhIG1ham9yIGNoYWxsZW5nZSBmYWNpbmcgcmVzZWFyY2hlcnMgYW5kIHdhdGVyIHJlc291cmNlIG1hbmFnZXJzLiBWYXJpb3VzIGFwcHJvYWNoZXMsIGluY2x1ZGluZyBudXRyaWVudCBsb2FkIHJlZHVjdGlvbiwgYXJ0aWZpY2lhbCBtaXhpbmcgYW5kIGZsdXNoaW5nLCBvbW5pdm9yb3VzIGZpc2ggcmVtb3ZhbCwgYWxnYWVjaWRlIGFwcGxpY2F0aW9ucyBhbmQgc2VkaW1lbnQgZHJlZGdpbmcsIGhhdmUgYmVlbiB1c2VkIHRvIHJlZHVjZSBibG9vbSBvY2N1cnJlbmNlcy4gSG93ZXZlciwgbWFuYWdlcnMgbm93IGZhY2UgdGhlIGFkZGl0aW9uYWwgY2hhbGxlbmdlIG9mIGhhdmluZyB0byBhZGRyZXNzIHRoZSBlZmZlY3RzIG9mIGNsaW1hdGUgY2hhbmdlIG9uIHdhdGVyc2hlZCBoeWRyb2xvZ2ljYWwgYW5kIG51dHJpZW50IGxvYWQgZHluYW1pY3MsIHdhdGVyIHRlbXBlcmF0dXJlLCBtaXhpbmcgcmVnaW1lIGFuZCBpbnRlcm5hbCBudXRyaWVudCBjeWNsaW5nLiBSaXNpbmcgdGVtcGVyYXR1cmVzIGFuZCBpbmNyZWFzaW5nIGZyZXF1ZW5jaWVzIGFuZCBtYWduaXR1ZGVzIG9mIGV4dHJlbWUgd2VhdGhlciBldmVudHMsIGluY2x1ZGluZyB0cm9waWNhbCBjeWNsb25lcywgZXh0cmF0cm9waWNhbCBzdG9ybXMsIGZsb29kcyBhbmQgZHJvdWdodHMsIGFsbCBwcm9tb3RlIEN5YW5vSEFCcyBhbmQgYWZmZWN0IHRoZSBlZmZpY2FjeSBvZiBlY29zeXN0ZW0gcmVtZWRpYXRpb24gbWVhc3VyZXMuIFRoZXNlIGNsaW1hdGljIGNoYW5nZXMgd2lsbCBsaWtlbHkgcmVxdWlyZSBzZXR0aW5nIHN0cmljdGVyIG51dHJpZW50IChpbmNsdWRpbmcgYm90aCBuaXRyb2dlbiBhbmQgcGhvc3Bob3J1cykgcmVkdWN0aW9uIHRhcmdldHMgZm9yIGJsb29tIGNvbnRyb2wgaW4gYWZmZWN0ZWQgd2F0ZXJzLiBJbiBhZGRpdGlvbiwgdGhlIGVmZmljYWN5IG9mIGN1cnJlbnRseSB1c2VkIG1ldGhvZHMgdG8gcmVkdWNlIEN5YW5vSEFCcyB3aWxsIG5lZWQgdG8gYmUgcmUtZXZhbHVhdGVkIGluIGxpZ2h0IG9mIHRoZSBzeW5lcmdpc3RpYyBlZmZlY3RzIG9mIGNsaW1hdGUgY2hhbmdlIHdpdGggbnV0cmllbnQgZW5yaWNobWVudC4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTA3MS9tZjE4MzkyIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2Nkci5saWIudW5jLmVkdS9kb3dubG9hZHMvMDU3NDIyODM0IiwiaXNzdWVkRGF0ZSI6IjIwMTktMDMtMjUiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTEwIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJNYXJpbmUgYW5kIEZyZXNod2F0ZXIgUmVzZWFyY2giLCJpc3N1ZSI6NSwidm9sdW1lIjo3MSwiaW1wYWN0RmFjdG9yIjoiMi43MCJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjU3OSwibGFzdFBhZ2UiOjU3OX19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJDU0lSTyBQdWJsaXNoaW5nIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjoxMDQsImNpdGVkQnlDb3VudCI6OTF9fSwicXVvdGUiOiJUaGF0IHJlbGF0aW9uc2hpcCBoYXMgc2VydmVkIGFzIHRoZSBmb3VuZGF0aW9uIGZvciBzZXR0aW5nICBudXRyaWVudCBsb2FkaW5nIHRhcmdldHMgaW4gbGFrZXMgYXJvdW5kIHRoZSB3b3JsZCwgYW5kIGNhcmVmdWxseSAgZGVzaWduZWQgZXhwZXJpbWVudHMgbWF5IGVsdWNpZGF0ZSB0byB3aGF0IGV4dGVudCB0ZW1wZXJhdHVyZSAgaW5jcmVhc2UgYWZmZWN0cyB0aGUgdGhyZXNob2xkIGF0IHdoaWNoIHRoZSBmcmVxdWVuY3kgb2Ygb2NjdXIgIHJlbmNlIG9mIEN5YW5vSEFCcyBxdWlja2x5IHJpc2VzIHdpdGggaW5jcmVhc2luZyBOIGFuZCBQLiBDb25jbHVzaW9ucyBSZWNlbnQgc3R1ZGllcyBpbmRpY2F0ZSB0aGF0IHRoZSBzeW5lcmdpc3RpYyBjb21iaW5hdGlvbiBvZiBhbnRocm9wb2dlbmljIG51dHJpZW50IGxvYWRpbmcsIHJpc2luZyB0ZW1wZXJhdHVyZXMsIGVuaGFuY2VkICB2ZXJ0aWNhbCBzdHJhdGlmaWNhdGlvbiwgaW5jcmVhc2VkIHJlc2lkZW5jZSB0aW1lIGFuZCBtb3JlICBleHRyZW1lIGNsaW1hdGljIGNvbmRpdGlvbnMgb3ZlcmFsbCB3aWxsIGZhdm91ciBjeWFub2JhY3RlcmlhbCAgZG9taW5hbmNlIGluIGEgd2lkZSByYW5nZSBvZiBhcXVhdGljIGVjb3N5c3RlbXMuIFRoZSBleHBhbiAgc2lvbiBvZiBDeWFub0hBQnMgaGFzIHNlcmlvdXMgY29uc2VxdWVuY2VzIGZvciBodW1hbiB3YXRlciAgc3VwcGxpZXMsIGZpc2hlcmllcyBhbmQgcmVjcmVhdGlvbmFsIHJlc291cmNlcy4gRWNvc3lzdGVtIHNjYWxlICBwaHlzaWNhbCBjaGVtaWNhbCBiaW90aWMgbWl0aWdhdGlvbiBzdHJhdGVnaWVzIHdpbGwgaGF2ZSB0byAgaW5jb3Jwb3JhdGUgbnV0cmllbnQgKG1vc3Qgb2Z0ZW4gYm90aCBOIGFuZCBQKSBpbnB1dCByZWR1Y3Rpb25zLiAgVGhlIG1hZ25pdHVkZXMgb2YgdGhlc2UgcmVkdWN0aW9ucyB3aWxsIG5lZWQgdG8gYmUgc3lzdGVtICBzcGVjaWZpYyBhbmQgd2lsbCBsaWtlbHkgbmVlZCB0byBiZSBhZGp1c3RlZCB3aXRoIGNoYW5naW5nIGNsaSAgbWF0aWMgY29uZGl0aW9ucy4gU3BlY2lmaWNhbGx5LCB3YXRlciBxdWFsaXR5IG1hbmFnZXJzIHdpbGwgaGF2ZSAgdG8gYWNjb21tb2RhdGUgd2FybWluZyBhbmQgZ3JlYXRlciBoeWRyb2xvZ2ljYWwgdmFyaWFiaWxpdHkgIGFuZCBleHRyZW1lbmVzcyByZXN1bHRpbmcgZnJvbSBjbGltYXRlIGNoYW5nZSBpbiBmb3JtdWxhdGluZyAgdGhlaXIgc3RyYXRlZ2llcy4gQSBrZXkgbG9uZyB0ZXJtIGNvbnRyb2wgd2UgY2FuIGV4ZXJ0IHRvIHJlZHVjZSAgdGhlIHJhdGUgYW5kIGV4dGVudCBvZiBnbG9iYWwgd2FybWluZyBpcyBjdXJiaW5nIGdyZWVuaG91c2UgZ2FzICBlbWlzc2lvbnMuIFdpdGhvdXQgdGhpcyBlc3NlbnRpYWwgc3RlcCwgZnV0dXJlIHdhcm1pbmcgdHJlbmRzLCAgaHlkcm9sb2dpY2FsIGV4dHJlbWVzIChyZWNvcmQgcmFpbmZhbGxzIGFuZCBkcm91Z2h0cykgYW5kIHRoZWlyICBlZmZlY3RzIG9uIGFxdWF0aWMgZWNvc3lzdGVtcyB3aWxsIGNvbnRpbnVlIHRvIHByb21vdGUgb3Bwb3IgIHR1bmlzdGljIEN5YW5vSEFCcy4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6MTQuMTQ5MjMzODE4MDU0MiwibGVmdCI6MTMuMTI4NTk1MzUyMTcyODUyLCJ3aWR0aCI6MzcuMDgyMTcyMzkzNzk4ODMsImhlaWdodCI6MS41MTY2MDIyNzc3NTU3MzczfX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjE1LjQ1MTUwODUyMjAzMzY5MSwibGVmdCI6MTMuMTI4NTQ0ODA3NDM0MDgyLCJ3aWR0aCI6MzcuMDg0ODMxMjM3NzkyOTcsImhlaWdodCI6MS41MTY2MDMyMzE0MzAwNTM3fX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjE2Ljc1Mzc4NDE3OTY4NzUsImxlZnQiOjEzLjEyODU0MjkwMDA4NTQ1LCJ3aWR0aCI6MzcuMDgzMjA5OTkxNDU1MDgsImhlaWdodCI6MS41MTY2MDQxODUxMDQzNzAxfX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjE4LjA1NjA1MzE2MTYyMTA5NCwibGVmdCI6MTMuMTI4NTQ0ODA3NDM0MDgyLCJ3aWR0aCI6MzYuNjE1MDc0MTU3NzE0ODQ0LCJoZWlnaHQiOjEuNTE2NjA0MTg1MTA0MzcwMX19LHsicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjoxOS40NjgzNzgwNjcwMTY2LCJsZWZ0IjoxMy4xMjg1NDQ4MDc0MzQwODIsIndpZHRoIjozNC4yOTA1MzExNTg0NDcyNjYsImhlaWdodCI6MS4yNDUwMTEyMTA0NDE1ODk0fX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjIyLjIyNjE0Mjg4MzMwMDc4LCJsZWZ0IjoxMy4xMjg1NTA1Mjk0Nzk5OCwid2lkdGgiOjcuNjQ5ODczNzMzNTIwNTA4LCJoZWlnaHQiOjEuMTM1NDgzMTQ1NzEzODA2Mn19LHsicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjoyMy43MzQ4NDk5Mjk4MDk1NywibGVmdCI6MTMuMTI4NTM1MjcwNjkwOTE4LCJ3aWR0aCI6MzcuMDg5Mjg2ODA0MTk5MjIsImhlaWdodCI6MjYuMDg4NjQwMjEzMDEyNjk1fX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNTg5In19XQ%3D%3D. In India, where lakes like Bellandur and Dal face nutrient overload from urbanization and agriculture, communities can implement these solutions using indigenous materials and knowledge, aligning with initiatives like Amrit Sarovars (Jain et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNodWJoYW0iLCJmYW1pbHkiOiJKYWluIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFtYW4iLCJmYW1pbHkiOiJTcml2YXN0YXZhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkRpbmVzaCBLdW1hciIsImZhbWlseSI6IlZpc2h3YWthcm1hIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkppdGVuZHJhIiwiZmFtaWx5IjoiUmFqcHV0In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5pdGluIExpbGFkaGFyIiwiZmFtaWx5IjoiUmFuZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBbGkgQXNnaGFyIiwiZmFtaWx5IjoiU2FsZW0ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQWhtZWQiLCJmYW1pbHkiOiJFbGJlbHRhZ2kifX1dLCJpZHMiOnsiaXNzbiI6WyIyNjI0LTkzNzUiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzODkvZnJ3YS4yMDI0LjE0NDEzNjUiLCJvcGVuYWxleCI6Ilc0NDAxOTI5NTc2In0sInRpdGxlIjoiUHJvdGVjdGluZyBhbmNpZW50IHdhdGVyIGhhcnZlc3RpbmcgdGVjaG5vbG9naWVzIGluIEluZGlhOiBzdHJhdGVnaWVzIGZvciBjbGltYXRlIGFkYXB0YXRpb24gYW5kIHN1c3RhaW5hYmxlIGRldmVsb3BtZW50IHdpdGggZ2xvYmFsIGxlc3NvbnMiLCJhYnN0cmFjdCI6IkludHJvZHVjdGlvbiBBbmNpZW50IHdhdGVyIGhhcnZlc3Rpbmcgc3lzdGVtcywgc3VjaCBhcyB0aG9zZSBmcm9tIHRoZSBJbmR1cyBWYWxsZXkgQ2l2aWxpemF0aW9uICh%2BMzUwMCBCQ0UpLCBoYXZlIGJlZW4gdml0YWwgZm9yIGlycmlnYXRpb24gYW5kIGNsaW1hdGUgcmVzaWxpZW5jZSwgZXNwZWNpYWxseSBpbiBhcmlkIHJlZ2lvbnMuIE9uZSBzdWNoIHByb21pbmVudCBzeXN0ZW0gaW4gU291dGggQXNpYSwgY2FsbGVkIHRhbmsgaXJyaWdhdGlvbiwgaW5pdGlhbGx5IHRocml2ZWQgdGhyb3VnaCBjb21tdW5pdHkgbWFuYWdlbWVudCBidXQgZGVjbGluZWQgcG9zdC1pbmRlcGVuZGVuY2UgZHVlIHRvIGNvbG9uaWFsIHBvbGljaWVzIGFuZCBuZWdsZWN0IGluIFNyaSBMYW5rYSBhbmQgSW5kaWEuIFRoaXMgc3R1ZHkgZXZhbHVhdGVzIGN1cnJlbnQgcG9saWN5IGZyYW1ld29ya3MgYW5kIHJlaGFiaWxpdGF0aW9uIHByb2dyYW1zIHRvIGVuaGFuY2UgdGhlIHJlc2lsaWVuY2Ugb2YgdGhlc2Ugc3lzdGVtcyBpbiBJbmRpYSwgZGV2ZWxvcCBzdHJhdGVnaWVzIGZvciB0aGVpciBwcm90ZWN0aW9uIGFuZCBhZGFwdGF0aW9uIHRvIGNsaW1hdGUgY2hhbmdlLCBhbmQgaW50ZWdyYXRlIGdsb2JhbCBsZXNzb25zIGZvciBzdXN0YWluYWJsZSBkZXZlbG9wbWVudC4gTWV0aG9kcyBBIHN5c3RlbWF0aWMgbWV0YS1hbmFseXNpcyBvZiBncmV5IGxpdGVyYXR1cmUgd2FzIGNvbmR1Y3RlZCB0byBhZ2dyZWdhdGUgZGF0YSBvbiBwb2xpY3kgY29uc3RyYWludHMuIFBvbGljeSBhbmFseXNpcyBpbnZvbHZlZCBkZXRhaWxlZCBpbnZlc3RpZ2F0aW9ucyBvZiByZWxldmFudCBkb2N1bWVudHMsIHJlZ3VsYXRpb25zLCBhbmQgY29tcGFyYXRpdmUgYW5hbHlzZXMgb2YgZnJhbWV3b3JrcyBhdCByZWdpb25hbCBhbmQgbmF0aW9uYWwgbGV2ZWxzLiBQaWxvdCBwcm9qZWN0cyBvbiB0YW5rIHJlaGFiaWxpdGF0aW9uIHdlcmUgYXNzZXNzZWQgdGhyb3VnaCByZXBvcnRlZCBjYXNlIHN0dWRpZXMgYW5kIGZpZWxkIHN1cnZleXMgdG8gZ2F1Z2UgaW1wYWN0LiBUaGVtYXRpYyBhbmFseXNpcyB3YXMgdXNlZCB0byBleHBsb3JlIHRoZSBnbG9iYWwgcG90ZW50aWFsIG9mIHRoZXNlIHN5c3RlbXMgaW4gY2xpbWF0ZSByZXNpbGllbmNlIGFuZCBvdmVyYWxsIGVudmlyb25tZW50YWwgc3VzdGFpbmFiaWxpdHkuIFJlc3VsdHMgVGhlIGFuYWx5c2lzIHNob3dlZCB0aGF0IHBpbG90IHByb2plY3RzIGZvciB0YW5rIHJlaGFiaWxpdGF0aW9uIGhhZCBsaW1pdGVkIHN1Y2Nlc3MgaW4gYWNoaWV2aW5nIHN1c3RhaW5hYmlsaXR5IHVuZGVyIGN1cnJlbnQgY2xpbWF0ZSBjb25kaXRpb25zLiBUYW5rIGlycmlnYXRpb24gc3lzdGVtcyBhcmUgY3J1Y2lhbCBmb3IgYWRhcHRpbmcgdG8gZXh0cmVtZSB3ZWF0aGVyLCBpbmNsdWRpbmcgZmxvb2RzLCBkcm91Z2h0cywgYW5kIGhlYXQgd2F2ZXMsIHJlcGxlbmlzaGluZyBncm91bmR3YXRlciwgcmVkdWNpbmcgc29pbCBlcm9zaW9uLCBhbmQgZW5zdXJpbmcgcmVsaWFibGUgd2F0ZXIgc3VwcGxpZXMuIFRyYWRpdGlvbmFsIHdhdGVyIGhhcnZlc3RpbmcgdGVjaG5vbG9naWVzIHN1cHBvcnQgMTcgU3VzdGFpbmFibGUgRGV2ZWxvcG1lbnQgR29hbHMgKFNER3MpLCBpbmNsdWRpbmcgY2xlYW4gd2F0ZXIgYWNjZXNzLCBodW5nZXIgcmVkdWN0aW9uLCBnZW5kZXIgZXF1YWxpdHksIGFuZCBjbGltYXRlIGFjdGlvbi4gSW50ZWdyYXRpbmcgQUkgYW5kIG1hY2hpbmUgbGVhcm5pbmcgaW4gd2F0ZXIgbWFuYWdlbWVudCBiZW5lZml0cyBkaXNhc3RlciByZXNwb25zZSwgd2hpbGUgZWNvLXRvdXJpc20gYWlkcyBzeXN0ZW0gbWFpbnRlbmFuY2UgYW5kIGN1bHR1cmFsIGF3YXJlbmVzcy4gRGlzY3Vzc2lvbiBUaGUgc3R1ZHkgdW5kZXJzY29yZXMgdGhlIG5lZWQgZm9yIHBvbGljeSByZWZvcm1zIHRvIGVuaGFuY2UgdGFuayByZWhhYmlsaXRhdGlvbiBhbmQgaW5zdGl0dXRpb25hbCBhcnJhbmdlbWVudHMuIEl0IGNhbGxzIGZvciBpbmNyZWFzZWQgYmVuZWZpY2lhcnkgcGFydGljaXBhdGlvbiBhbmQgY29uc3RpdHV0aW9uYWwgcmVjb2duaXRpb24gb2YgY3VycmVudCBwcmFjdGljZXMuIFN0cmF0ZWdpYywgbmF0aW9uYWwtc2NhbGUgYXNzZXNzbWVudHMgYW5kIHJlc2lsaWVuY2UgdGFyZ2V0cyBhcmUgcmVjb21tZW5kZWQgdG8gaW1wcm92ZSB0aGUgZWZmZWN0aXZlbmVzcyBvZiBzdWNoIHdhdGVyIGhhcnZlc3Rpbmcgc3lzdGVtcyBpbiBtaXRpZ2F0aW5nIG5hdHVyYWwgaGF6YXJkcyBhbmQgZW5oYW5jaW5nIGVudmlyb25tZW50YWwgc2VydmljZXMuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjMzODkvZnJ3YS4yMDI0LjE0NDEzNjUiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzg5L2Zyd2EuMjAyNC4xNDQxMzY1IiwiaXNzdWVkRGF0ZSI6IjIwMjQtMDgtMjciLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTExIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJGcm9udGllcnMgaW4gV2F0ZXIiLCJ2b2x1bWUiOjYsImltcGFjdEZhY3RvciI6IjMuOTMifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRnJvbnRpZXJzIE1lZGlhIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo4OCwiY2l0ZWRCeUNvdW50IjoxNn19LCJxdW90ZSI6IlNlY3Rpb24gVGl0bGU6IFByb3RlY3RpbmcgYW5jaWVudCB3YXRlciBoYXJ2ZXN0aW5nIHRlY2hub2xvZ2llcyBpbiBJbmRpYTogc3RyYXRlZ2llcyBmb3IgY2xpbWF0ZSBhZGFwdGF0aW9uIGFuZCAuLi5cbkNvbnRlbnQ6XG4qKkludHJvZHVjdGlvbjoqKiBBbmNpZW50IHdhdGVyIGhhcnZlc3Rpbmcgc3lzdGVtcywgc3VjaCBhcyB0aG9zZSBmcm9tIHRoZSBJbmR1cyBWYWxsZXkgQ2l2aWxpemF0aW9uICh%2BMzUwMCBCQ0UpLCBoYXZlIGJlZW4gdml0YWwgZm9yIGlycmlnYXRpb24gYW5kIGNsaW1hdGUgcmVzaWxpZW5jZSwgZXNwZWNpYWxseSBpbiBhcmlkIHJlZ2lvbnMuIE9uZSBzdWNoIHByb21pbmVudCBzeXN0ZW0gaW4gU291dGggQXNpYSwgY2FsbGVkIHRhbmsgaXJyaWdhdGlvbiwgaW5pdGlhbGx5IHRocml2ZWQgdGhyb3VnaCBjb21tdW5pdHkgbWFuYWdlbWVudCBidXQgZGVjbGluZWQgcG9zdC1pbmRlcGVuZGVuY2UgZHVlIHRvIGNvbG9uaWFsIHBvbGljaWVzIGFuZCBuZWdsZWN0IGluIFNyaSBMYW5rYSBhbmQgSW5kaWEuIFRoaXMgc3R1ZHkgZXZhbHVhdGVzIGN1cnJlbnQgcG9saWN5IGZyYW1ld29ya3MgYW5kIHJlaGFiaWxpdGF0aW9uIHByb2dyYW1zIHRvIGVuaGFuY2UgdGhlIHJlc2lsaWVuY2Ugb2YgdGhlc2Ugc3lzdGVtcyBpbiBJbmRpYSwgZGV2ZWxvcCBzdHJhdGVnaWVzIGZvciB0aGVpciBwcm90ZWN0aW9uIGFuZCBhZGFwdGF0aW9uIHRvIGNsaW1hdGUgY2hhbmdlLCBhbmQgaW50ZWdyYXRlIGdsb2JhbCBsZXNzb25zIGZvciBzdXN0YWluYWJsZSBkZXZlbG9wbWVudC4ifV0%3D. By fostering self-reliance, NBS reduce reliance on often-corrupt regulatory bodies, generating verifiable improvements in water quality while reviving cultural stewardship(Everard et al., 2017)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1hcmsiLCJmYW1pbHkiOiJFdmVyYXJkIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik8uIFAuIiwiZmFtaWx5IjoiU2hhcm1hIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlZpbm9kIEt1bWFyIiwiZmFtaWx5IjoiVmlzaHdha2FybWEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiRGhhcm1lbmRyYSIsImZhbWlseSI6IktoYW5kYWwifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiWW9nZXNoIiwiZmFtaWx5IjoiU2FodSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJSYWh1bCIsImZhbWlseSI6IkJoYXRuYWdhciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJKaXRlbmRyYSBLdW1hciIsImZhbWlseSI6IlNpbmdoIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlJpdGVzaCIsImZhbWlseSI6Ikt1bWFyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFzZ2hhciIsImZhbWlseSI6Ik5hd2FiIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFtaXQiLCJmYW1pbHkiOiJLdW1hciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJWaXZlayIsImZhbWlseSI6Ikt1bWFyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFuaWwiLCJmYW1pbHkiOiJLYXNoeWFwIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkRlZXAgTmFyYXlhbiIsImZhbWlseSI6IlBhbmRleSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBZHJpYW4gQy4iLCJmYW1pbHkiOiJQaW5kZXIifX1dLCJpZHMiOnsiaXNzbiI6WyIwMDQ4LTk2OTciLCIxODc5LTEwMjYiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMTYvai5zY2l0b3RlbnYuMjAxNy4wOC4zMDgiLCJtYWciOiIyNzUzNDM1NzkxIiwicG1pZCI6IjI4ODkyODY4Iiwib3BlbmFsZXgiOiJXMjc1MzQzNTc5MSJ9LCJ0aXRsZSI6IkFzc2Vzc2luZyB0aGUgZmVhc2liaWxpdHkgb2YgaW50ZWdyYXRpbmcgZWNvc3lzdGVtLWJhc2VkIHdpdGggZW5naW5lZXJlZCB3YXRlciByZXNvdXJjZSBnb3Zlcm5hbmNlIGFuZCBtYW5hZ2VtZW50IGZvciB3YXRlciBzZWN1cml0eSBpbiBzZW1pLWFyaWQgbGFuZHNjYXBlczogQSBjYXNlIHN0dWR5IGluIHRoZSBCYW5hcyBjYXRjaG1lbnQsIFJhamFzdGhhbiwgSW5kaWEiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAxNi9qLnNjaXRvdGVudi4yMDE3LjA4LjMwOCIsInJlc291cmNlVXJsIjoiaHR0cDovL2VwcmludHMuYm91cm5lbW91dGguYWMudWsvMjk2ODUvMS8yMDE3LTA4LTI0JTIwcmVzdWJtaXR0ZWQlMjBwYXBlciUyMFNUT1RFTi1ELTE3LTA1MzM5UjIucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMTctMDktMDgiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA3IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJUaGUgU2NpZW5jZSBvZiBUaGUgVG90YWwgRW52aXJvbm1lbnQiLCJ2b2x1bWUiOjYxMiwiaW1wYWN0RmFjdG9yIjoiMTEuOTQifSwiY29udGFpbmVyTG9jYXRvciI6eyJwYWdlUmFuZ2UiOnsiZmlyc3RQYWdlIjoxMjQ5LCJsYXN0UGFnZSI6MTI2NX19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJFbHNldmllciBCViJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NjEsImNpdGVkQnlDb3VudCI6Njh9fSwicXVvdGUiOiJUaGVyZSBpcyBhIGdyb3dpbmcgbGl0ZXJhdHVyZSA2NTIgYXNzZXJ0aW5nIHRoYXQgdHJhZGl0aW9uYWwga25vd2xlZGdlLCBjdXJyZW50bHkgYmVpbmcgbG9zdCB0aHJvdWdoIHZpbGxhZ2UgNjUzIGFiYW5kb25tZW50IGFuZCBjb252ZXJzaW9uIHRvIG1lY2hhbmlzZWQgdGVjaG5pcXVlcywgY2FuIHBsYXkgc2lnbmlmaWNhbnQgcm9sZXMgaW4gNjU0IHJlYmFsYW5jaW5nIHdhdGVyIHJlc291cmNlIHJlY2hhcmdlIHdpdGggZGVtYW5kcyBvbiByZWNlZGluZyBncm91bmR3YXRlciBpZiA2NTUgYXBwcm9wcmlhdGVseSBzdXBwb3J0ZWQgYnkgcmVmb3JtZWQgcG9saWNpZXMgYW5kIGludmVzdG1lbnQgKGZvciBleGFtcGxlIFNoYWggYW5kIDY1NiBSYWp1LCAyMDAyOyBQYW5kZXkgZXQgYWwuLCAyMDAzOyBSYXRob3JlLCAyMDA1OyBOYXJhaW4gZXQgYWwuLCAyMDA1OyBFdmVyYXJkLCAyMDE1KS4gNjU3IFdhdGVyc2hlZCBtYW5hZ2VtZW50IHByb2dyYW1tZXMgcHJvbW90aW5nIHRoZSBkaXN0cmlidXRlZCByZXN0b3JhdGlvbiBvZiBzbWFsbC02NTggc2NhbGUgd2F0ZXIgaGFydmVzdGluZyBoYXZlIHJlc3VsdGVkIGluIHNpZ25pZmljYW50IGltcGFjdHMgb24gY2F0Y2htZW50IGh5ZHJvbG9neSA2NTkgYW5kIGRvd25zdHJlYW0gd2F0ZXIgYXZhaWxhYmlsaXR5IGluIEFuZGhyYSBQcmFkZXNoIGFuZCBvdGhlciBwYXJ0cyBvZiBJbmRpYSAoRkFPLCA2NjAgMjAxMikuIFNpZ25pZmljYW50IGdyb3VuZHdhdGVyIHJpc2VzIGFyZSByZXBvcnRlZCB3aGVyZSBjb21tdW5pdHktYmFzZWQgNjYxIHBhcnRpY2lwYXRvcnkgbWV0aG9kcyBoYXZlIGJlZW4gZGV2ZWxvcGVkIGF0IGJlbmNobWFyayBzaXRlcyBpbiBzZXZlcmFsIEluZGlhbiA2NjIgc3RhdGVzL3Byb3ZpbmNlcyBhbW9uZ3N0IGEgd2lkZSByYW5nZSBvZiBleHBlcmltZW50YWwgd2F0ZXJzaGVkcyBhY3Jvc3MgQXNpYSA2NjMgKFdhbmkgZXQgYWwuLCAyMDAzLCAyMDA1LCAyMDA2IGFuZCAyMDA5KS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjY5LCJiYm94Ijp7InRvcCI6NjMuODE2MDM2MjI0MzY1MjM0LCJsZWZ0IjoxMi4wOTgzNjc2OTEwNDAwMzksIndpZHRoIjo3My45NjA1NjM2NTk2Njc5NywiaGVpZ2h0IjoxLjk1ODM4NTM0ODMyMDAwNzN9fSx7InBhZ2VOdW1iZXIiOjY5LCJiYm94Ijp7InRvcCI6NjQuMzY0Nzg0MjQwNzIyNjYsImxlZnQiOjYuMjI4NTgzMzM1ODc2NDY1LCJ3aWR0aCI6My4yNjE0MTU3MTk5ODU5NjIsImhlaWdodCI6MS4zMTEyOTMxMjUxNTI1ODh9fSx7InBhZ2VOdW1iZXIiOjY5LCJiYm94Ijp7InRvcCI6NjUuNjQwNDM0MjY1MTM2NzIsImxlZnQiOjYuMjI4NTgzMzM1ODc2NDY1LCJ3aWR0aCI6NjkuNjk3NjM5NDY1MzMyMDMsImhlaWdodCI6MS45NTgzODUzNDgzMjAwMDczfX0seyJwYWdlTnVtYmVyIjo2OSwiYmJveCI6eyJ0b3AiOjY3LjQzNjMzMjcwMjYzNjcyLCJsZWZ0Ijo2LjIyODU4MzMzNTg3NjQ2NSwid2lkdGgiOjgyLjE1NDEyMTM5ODkyNTc4LCJoZWlnaHQiOjEuOTU4Mzg1MzQ4MzIwMDA3M319LHsicGFnZU51bWJlciI6NjksImJib3giOnsidG9wIjo2OS4yNDY0NzUyMTk3MjY1NiwibGVmdCI6Ni4yMjg1ODMzMzU4NzY0NjUsIndpZHRoIjo3Ni42NjU5ODUxMDc0MjE4OCwiaGVpZ2h0IjoxLjk1ODM4NTM0ODMyMDAwNzN9fSx7InBhZ2VOdW1iZXIiOjY5LCJiYm94Ijp7InRvcCI6NzEuMDQyMzgxMjg2NjIxMSwibGVmdCI6Ni4yMjg1ODMzMzU4NzY0NjUsIndpZHRoIjo4MS4zMjAzMjAxMjkzOTQ1MywiaGVpZ2h0IjoxLjk1ODM4NTM0ODMyMDAwNzN9fSx7InBhZ2VOdW1iZXIiOjY5LCJiYm94Ijp7InRvcCI6NzIuODM4Mjc5NzI0MTIxMSwibGVmdCI6Ni4yMjg1ODMzMzU4NzY0NjUsIndpZHRoIjo4MS4xNjU3NTYyMjU1ODU5NCwiaGVpZ2h0IjoxLjk1ODM4NTM0ODMyMDAwNzN9fSx7InBhZ2VOdW1iZXIiOjY5LCJiYm94Ijp7InRvcCI6NzUuNTg5MTI2NTg2OTE0MDYsImxlZnQiOjYuMjI4NTgzMzM1ODc2NDY1LCJ3aWR0aCI6ODAuODczNDY2NDkxNjk5MjIsImhlaWdodCI6MS45NTgzODUzNDgzMjAwMDczfX0seyJwYWdlTnVtYmVyIjo2OSwiYmJveCI6eyJ0b3AiOjc3LjIyODIzMzMzNzQwMjM0LCJsZWZ0Ijo2LjIyODU4MzMzNTg3NjQ2NSwid2lkdGgiOjgwLjI0MDIxMTQ4NjgxNjQsImhlaWdodCI6MS45NTgzODUzNDgzMjAwMDczfX0seyJwYWdlTnVtYmVyIjo2OSwiYmJveCI6eyJ0b3AiOjc4Ljg2NzM0NzcxNzI4NTE2LCJsZWZ0Ijo2LjIyODU4MzMzNTg3NjQ2NSwid2lkdGgiOjgxLjEyNDc3ODc0NzU1ODYsImhlaWdodCI6MS45NTgzODUzNDgzMjAwMDczfX0seyJwYWdlTnVtYmVyIjo2OSwiYmJveCI6eyJ0b3AiOjgwLjUwNjQ1NDQ2Nzc3MzQ0LCJsZWZ0Ijo2LjIyODU4MzMzNTg3NjQ2NSwid2lkdGgiOjczLjIxNTc1MTY0Nzk0OTIyLCJoZWlnaHQiOjEuOTU4Mzg1MzQ4MzIwMDA3M319LHsicGFnZU51bWJlciI6NjksImJib3giOnsidG9wIjo4Mi4xNDU1Njg4NDc2NTYyNSwibGVmdCI6Ni4yMjg1ODMzMzU4NzY0NjUsIndpZHRoIjo3OC4yMDgwMjMwNzEyODkwNiwiaGVpZ2h0IjoxLjk1ODM4NTM0ODMyMDAwNzN9fSx7InBhZ2VOdW1iZXIiOjY5LCJiYm94Ijp7InRvcCI6ODMuNzg3MDYzNTk4NjMyODEsImxlZnQiOjYuMjI4NTgzMzM1ODc2NDY1LCJ3aWR0aCI6NzcuNTM2NzczNjgxNjQwNjIsImhlaWdodCI6MS45NTgzODUzNDgzMjAwMDczfX0seyJwYWdlTnVtYmVyIjo2OSwiYmJveCI6eyJ0b3AiOjg1LjQxMTkxMTAxMDc0MjE5LCJsZWZ0IjoxMi4wOTgzNjc2OTEwNDAwMzksIndpZHRoIjo3Mi41ODU0OTQ5OTUxMTcxOSwiaGVpZ2h0IjoxLjk1ODM4NTM0ODMyMDAwNzN9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIxMzE4In19XQ%3D%3D.

Floating treatment wetlands represent a flagship NBS, where buoyant rafts planted with native macrophytes like Eichhornia crassipes or Typha latifolia float on lake surfaces to phytoremediate excess nitrogen and phosphorus. Communities can construct FTWs using recycled plastic bottles, bamboo frames, and local sediments, anchoring them in shallow bays prone to Microcystis dominance. These systems uptake nutrients via root biofilms, outcompeting algae and oxygenating hypoxic depths to curb internal loading (Jansen et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkpvYWNoaW0iLCJmYW1pbHkiOiJKYW5zZW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiR2F2aW4gTC4iLCJmYW1pbHkiOiJTaW1wc29uIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikdlc2EgQS4iLCJmYW1pbHkiOiJXZXloZW5tZXllciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJMYXVyYSBILiIsImZhbWlseSI6IkjDpHJrw7ZuZW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW5kcmV3IE0uIiwiZmFtaWx5IjoiUGF0ZXJzb24ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUGF1bCBBLiBkZWwiLCJmYW1pbHkiOiJHaW9yZ2lvIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ill2ZXMgVC4iLCJmYW1pbHkiOiJQcmFpcmllIn19XSwiaWRzIjp7Imlzc24iOlsiMTc1OC02NzhYIiwiMTc1OC02Nzk4Il0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDM4L3M0MTU1OC0wMjQtMDIwNTgtMyIsIm9wZW5hbGV4IjoiVzQ0MDAyMDgyMDAiLCJleGEiOiJodHRwczovL3d3dy5uYXR1cmUuY29tL2FydGljbGVzL3M0MTU1OC0wMjQtMDIwNTgtMz9lcnJvcj1jb29raWVzX25vdF9zdXBwb3J0ZWQmY29kZT1hNzRhMjVmZi1hNThkLTRkZjgtOTM2Zi0yNjJlMmUzZTkxOTgifSwidGl0bGUiOiJDbGltYXRlLWRyaXZlbiBkZW94eWdlbmF0aW9uIG9mIG5vcnRoZXJuIGxha2VzIiwiYWJzdHJhY3QiOiJBYnN0cmFjdCBPeHlnZW4gZGVwbGV0aW9uIGNvbnN0aXR1dGVzIGEgbWFqb3IgdGhyZWF0IHRvIGxha2UgZWNvc3lzdGVtcyBhbmQgdGhlIHNlcnZpY2VzIHRoZXkgcHJvdmlkZS4gTW9zdCBvZiB0aGUgd29ybGTigJlzIGxha2VzIGFyZSBsb2NhdGVkICZndDs0NcKwIE4sIHdoZXJlIGFjY2VsZXJhdGVkIGNsaW1hdGUgd2FybWluZyBhbmQgZWxldmF0ZWQgY2FyYm9uIGxvYWRzIG1pZ2h0IHNldmVyZWx5IGluY3JlYXNlIHRoZSByaXNrIG9mIGh5cG94aWEsIGJ1dCB0aGlzIGhhcyBub3QgYmVlbiBzeXN0ZW1hdGljYWxseSBleGFtaW5lZC4gSGVyZSBhbmFseXNpcyBvZiAyLjYgbWlsbGlvbiB3YXRlciBxdWFsaXR5IG9ic2VydmF0aW9ucyBmcm9tIDgsMjg4IGxha2VzIHNob3dzIHRoYXQgYmV0d2VlbiAxOTYwIGFuZCAyMDIyLCBtb3N0IG5vcnRoZXJuIGxha2VzIGV4cGVyaWVuY2VkIHJhcGlkIGRlb3h5Z2VuYXRpb24gc3Ryb25nbHkgbGlua2VkIHRvIGNsaW1hdGUtZHJpdmVuIHByb2xvbmdhdGlvbiBvZiBzdW1tZXIgc3RyYXRpZmljYXRpb24uIE94eWdlbiBsZXZlbHMgZGV0ZXJpb3JhdGVkIG1vc3QgaW4gc21hbGwgbGFrZXMgKCZsdDsxMCBoYSkgb3dpbmcgdG8gdGhlaXIgZ3JlYXRlciB2b2x1bWV0cmljIG94eWdlbiBkZW1hbmQgYW5kIHN1cmZhY2Ugd2FybWluZyByYXRlcywgd2hpbGUgdGhlIGxhcmdlc3QgbGFrZXMgZ2FpbmVkIG94eWdlbiB1bmRlciBtaW5pbWFsIHN0cmF0aWZpY2F0aW9uIGNoYW5nZXMgYW5kIGltcHJvdmVkIGFlcmF0aW9uIGF0IHNwcmluZyBvdmVydHVybnMuIFNlYXNvbmFsIG94eWdlbiBjb25zdW1wdGlvbiByYXRlcyBkZWNsaW5lZCwgZGVzcGl0ZSB3aWRlc3ByZWFkIGJyb3duaW5nLiBQcm9saWZlcmF0aW5nIGFub3hpYSBlbmhhbmNlZCBzZWFzb25hbCBpbnRlcm5hbCBsb2FkaW5nIG9mIEMsIFAgYW5kIE4gYnV0IGRlcGxldGVkIFAgbG9uZy10ZXJtLCBpbmRpY2F0aW5nIHRoYXQgZGVveHlnZW5hdGlvbiBjYW4gZXhoYXVzdCByZWRveC1zZW5zaXRpdmUgZnJhY3Rpb25zIG9mIHNlZGltZW50IG51dHJpZW50IHJlc2Vydm9pcnMuIiwidXJsIjoiaHR0cHM6Ly9kb2kub3JnLzEwLjEwMzgvczQxNTU4LTAyNC0wMjA1OC0zIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAzOC9zNDE1NTgtMDI0LTAyMDU4LTMiLCJpc3N1ZWREYXRlIjoiMjAyNC0wNy0wMSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTEiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6Ik5hdHVyZSBDbGltYXRlIENoYW5nZSIsImlzc3VlIjo4LCJ2b2x1bWUiOjE0LCJpbXBhY3RGYWN0b3IiOiIxNS4wMiJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjgzMiwibGFzdFBhZ2UiOjgzOH19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJOYXR1cmUgUG9ydGZvbGlvIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo4MCwiY2l0ZWRCeUNvdW50IjozN319LCJxdW90ZSI6IkFlcm9iaWMgcmVzcGlyYXRpb24gY2FuIGRlcGxldGUgb3h5Z2VuIGNvbmNlbnRyYXRpb25zIGF0IGRlcHRoIHdoZW4gc2Vhc29uYWwgc3RyYXRpZmljYXRpb24gcHJldmVudHMgYWVyYXRpb24gb2YgdGhlIHdhdGVyIGNvbHVtbls1XShodHRwczovL3d3dy5uYXR1cmUuY29tL2FydGljbGVzL3M0MTU1OC0wMjQtMDIwNTgtMyNyZWYtQ1I1KS4gVGhpcyBkZXRyaW1lbnRhbCBjb25kaXRpb24sIHRlcm1lZCBoeXBveGlhIChET%2BKAiTzigIky4oCJbWfigIls4oiSMSksIGRpbWluaXNoZXMgdGhlIGNvbGQtd2F0ZXIgaGFiaXRhdCBvZiBhcXVhdGljIGxpZmUsIGluY2x1ZGluZyBjb21tZXJjaWFsbHkgaW1wb3J0YW50IGZpc2hbNl0oaHR0cHM6Ly93d3cubmF0dXJlLmNvbS9hcnRpY2xlcy9zNDE1NTgtMDI0LTAyMDU4LTMjcmVmLUNSNikgYW5kIGNhbiBjYXVzZSBtYXNzIG1vcnRhbGl0eSBpZiBwZXJzaXN0ZW50WzddKGh0dHBzOi8vd3d3Lm5hdHVyZS5jb20vYXJ0aWNsZXMvczQxNTU4LTAyNC0wMjA1OC0zI3JlZi1DUjcpLiBBbm94aWEgKERP4oCJPOKAiTAuNeKAiW1n4oCJbOKIkjEpIHByb21vdGVzIHRoZSByZWR1Y3RpdmUgZGlzc29sdXRpb24gb2YgbWluZXJhbC1ib3VuZCBjYXJib24gYW5kIHBob3NwaG9ydXNbOF0oaHR0cHM6Ly93d3cubmF0dXJlLmNvbS9hcnRpY2xlcy9zNDE1NTgtMDI0LTAyMDU4LTMjcmVmLUNSOCksIFs5XShodHRwczovL3d3dy5uYXR1cmUuY29tL2FydGljbGVzL3M0MTU1OC0wMjQtMDIwNTgtMyNyZWYtQ1I5KSB3aGljaCBmdWVscyBoYXJtZnVsIGFsZ2FsIGJsb29tc1sxMF0oaHR0cHM6Ly93d3cubmF0dXJlLmNvbS9hcnRpY2xlcy9zNDE1NTgtMDI0LTAyMDU4LTMjcmVmLUNSMTApIGFuZCBsaW1pdHMgbWljcm9iaWFsIG94aWRhdGlvbiBvZiBtZXRoYW5lLCBhIHBvdGVudCBncmVlbmhvdXNlIGdhc1sxMV0oaHR0cHM6Ly93d3cubmF0dXJlLmNvbS9hcnRpY2xlcy9zNDE1NTgtMDI0LTAyMDU4LTMjcmVmLUNSMTEpLiJ9XQ%3D%3D. Pilots in Loktak Lake demonstrate 30-50% phosphorus reduction within months, with harvested plants convertible to compost or biogas, offsetting costs (Nõges et al., 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRpaW5hIiwiZmFtaWx5IjoiTsO1Z2VzIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hc2ltZSIsImZhbWlseSI6IkphbmF0aWFuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlJlZXQiLCJmYW1pbHkiOiJMYXVnYXN0ZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQZWV0ZXIiLCJmYW1pbHkiOiJOw7VnZXMifX1dLCJpZHMiOnsiaXNzbiI6WyIyMzUxLTk4OTQiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMTYvai5nZWNjby4yMDIwLmUwMTM2OSIsIm1hZyI6IjMxMDU0NDYwOTAiLCJvcGVuYWxleCI6IlczMTA1NDQ2MDkwIiwiZXhhIjoiaHR0cHM6Ly93d3cuc2NpZW5jZWRpcmVjdC5jb20vc2NpZW5jZS9hcnRpY2xlL3BpaS9TMjM1MTk4OTQyMDMwOTEwMCJ9LCJ0aXRsZSI6IlBvc3Qtc292aWV0IGNoYW5nZXMgaW4gbml0cm9nZW4gYW5kIHBob3NwaG9ydXMgc3RvaWNoaW9tZXRyeSBpbiB0d28gbGFyZ2Ugbm9uLXN0cmF0aWZpZWQgbGFrZXMgYW5kIHRoZSBpbXBhY3Qgb24gcGh5dG9wbGFua3RvbiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2ouZ2VjY28uMjAyMC5lMDEzNjkiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2ouZ2VjY28uMjAyMC5lMDEzNjkiLCJpc3N1ZWREYXRlIjoiMjAyMC0xMS0yMCIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTEiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6Ikdsb2JhbCBFY29sb2d5IGFuZCBDb25zZXJ2YXRpb24iLCJ2b2x1bWUiOjI0LCJpbXBhY3RGYWN0b3IiOiIzLjU5In0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkVsc2V2aWVyIEJWIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo3MSwiY2l0ZWRCeUNvdW50IjoxNH19LCJxdW90ZSI6IlJhbmRvbSBGb3Jlc3QgYW5hbHlzaXMgaW5kaWNhdGVkIGFuIGltcG9ydGFudCByb2xlIG9mIGxpZ2h0IGxpbWl0YXRpb24gaW4gYm90aCBsYWtlcy4gQ29uc3RhbnRseSBsb3dlciBsZXZlbHMgb2YgTiBpbiB0aGUgZGVlcGVyIExha2UgUGVpcHNpIGZhdm91cmVkIE4yLWZpeGluZyBzcGVjaWVzLCB3aGljaCwgYXMgYSBwYXJhZG94LCBiZWNhbWUgUC1saW1pdGVkLiBBcyBjbGltYXRlIHdhcm1pbmcgcmVpbmZvcmNlcyBldXRyb3BoaWNhdGlvbiBwaGVub21lbmEgaW4gbGFrZXMgYnkgaW5jcmVhc2luZyBpbnRlcm5hbCBudXRyaWVudCBsb2FkaW5nIGFuZCBmYXZvdXJpbmcgYmxvb20tZm9ybWluZyBjeWFub2JhY3RlcmlhLCBtb3JlIHN0cmluZ2VudCBtZWFzdXJlcyB3b3VsZCBiZSBuZWVkZWQgdG8gZnVydGhlciBsaW1pdCBudXRyaWVudCBsb2FkcyAoZXNwZWNpYWxseSB0aGF0IG9mIFApIHRvIGxha2VzIHRocm91Z2ggaW1wcm92ZWQgd2FzdGV3YXRlciB0cmVhdG1lbnQgYW5kIGluY3JlYXNlZCBlZmZpY2llbmN5IG9mIGZlcnRpbGlzZXIgYXBwbGljYXRpb24uIExvbmctdGVybSBjaGFuZ2VzIGluIG51dHJpZW50IGxvYWRpbmcgYW5kIHN0b2ljaGlvbWV0cnkgaW4gd2F0ZXIgYm9kaWVzIHJlZmxlY3QgY2hhbmdlcyBpbiBhZ3JpY3VsdHVyYWwgcHJhY3RpY2VzIGFuZCB3YXN0ZXdhdGVyIHRyZWF0bWVudC4ifV0%3D. Integrating TEK, such as Meitei phumdi management, enhances efficacy by timing harvests to monsoon cycles, preventing secondary pollution (Nair & Nayak, 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5hdnlhIFZpa3JhbWFuIiwiZmFtaWx5IjoiTmFpciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQcmF0ZWVwIEt1bWFyIiwiZmFtaWx5IjoiTmF5YWsifX1dLCJpZHMiOnsiaXNzbiI6WyIyMjk2LTc3NDUiXSwiaXNibiI6W10sImRvaSI6IjEwLjMzODkvZm1hcnMuMjAyMy4xMDg3Mjk2Iiwib3BlbmFsZXgiOiJXNDM4MjcyMzEzMyJ9LCJ0aXRsZSI6IlVuY292ZXJpbmcgd2F0ZXIgcXVhbGl0eSBhbmQgZXZhbHVhdGluZyB2dWxuZXJhYmlsaXRpZXMgb2Ygc21hbGwtc2NhbGUgZmlzaGVyaWVzIGluIENoaWxpa2EgTGFnb29uLCBJbmRpYSIsImFic3RyYWN0IjoiU21hbGwtc2NhbGUgZmlzaGVyaWVzIChTU0ZzKSBwbGF5IGEgc3Ryb25nIHJvbGUgaW4gc3VzdGFpbmluZyBtaWxsaW9ucyBvZiBsaXZlbGlob29kcywgZm9vZCBzZWN1cml0eSwgbnV0cml0aW9uLCBhbmQgaW5jb21lIGdsb2JhbGx5IGJ1dCB0aGUgZmlzaGVycyBlbmdhZ2VkIGluIHRoaXMgc2VjdG9yIHNpbXVsdGFuZW91c2x5IGV4cGVyaWVuY2UgaGlnaCBsZXZlbHMgb2YgdnVsbmVyYWJpbGl0eSBhbmQgcHJvY2Vzc2VzIG9mIG1hcmdpbmFsaXNhdGlvbi4gU2V2ZXJhbCBmYWN0b3JzIGFyZSBhdHRyaWJ1dGVkIHRvIHRoZSBtdWx0aWRpbWVuc2lvbmFsIHZ1bG5lcmFiaWxpdGllcyB0aGUgc21hbGwtc2NhbGUgZmlzaGVycyBleHBlcmllbmNlLCBzcGFubmluZyBib3RoIG5hdHVyYWwgKGUuZy4sIG5hdHVyYWwgZGlzYXN0ZXJzLCBlY29zeXN0ZW0gY2hhbmdlKSBhbmQgYW50aHJvcG9nZW5pYyAoZS5nLiwgcG9saWN5IGNoYW5nZSwgaHlkcm9sb2dpY2FsIGludGVydmVudGlvbnMsIGFxdWFjdWx0dXJlKSBwcmVzc3VyZXMuIFdoaWxlIHRoZXJlIGlzIG11Y2ggbGl0ZXJhdHVyZSBvbiB2YXJpb3VzIG5hdHVyYWwgYW5kIGh1bWFuIGRyaXZlcnMgb2YgdnVsbmVyYWJpbGl0eSBpbiBzbWFsbC1zY2FsZSBmaXNoZXJ5IGNvbW11bml0aWVzLCBhbiBhYnNlbmNlIG9mIHJlc2VhcmNoIGNvbm5lY3RpbmcgdnVsbmVyYWJpbGl0eSB3aXRoIHdhdGVyIHF1YWxpdHkgaXMgZXZpZGVudC4gRmlzaGVyIGNvbW11bml0aWVzIG9mdGVuIHRhbGsgYWJvdXQgZmlzaCBpbiByZWxhdGlvbiB0byB0aGUgaGVhbHRoIG9mIHRoZWlyIGFxdWF0aWMgaGFiaXRhdHMgd2hlcmVpbiB3YXRlciBxdWFsaXR5IGlzIHNlZW4gYXMgYSBrZXkgcGFyYW1ldGVyLiBUaGUgbGluayBiZXR3ZWVuIGhlYWx0aHkgZmlzaCBhbmQgZ29vZCBxdWFsaXR5IHdhdGVyIGhhcyBzaWduaWZpY2FudCBpbXBsaWNhdGlvbnMgZm9yIHN0cm9uZyBhbmQgdmlhYmxlIGZpc2hpbmcgY29tbXVuaXRpZXMuIFRoaXMgcGFwZXIgZXhhbWluZXMgdGhlc2UgbGlua3MgZnVydGhlciBieSBmb2N1c2luZyBvbiB0aGUgbmF0dXJlIG9mIHZ1bG5lcmFiaWxpdGllcyBjYXVzZWQgYnkgd2F0ZXIgcXVhbGl0eSBjaGFuZ2VzIGluIHRoZSBzbWFsbC1zY2FsZSBmaXNoZXJ5IHN5c3RlbSBvZiBDaGlsaWthIExhZ29vbiBpbiBJbmRpYS4gV2UgdW5kZXJ0YWtlIGRldGFpbGVkIGFuYWx5c2lzIG9mIHRoZSBpbnZhc2l2ZSBzaHJpbXAgYXF1YWN1bHR1cmUgYWN0aXZpdGllcyBhbmQgaHlkcm9sb2dpY2FsIGludGVydmVudGlvbnMgZm9yIG9wZW5pbmcgb2YgYSBsYWdvb24gaW5sZXQgd2l0aCB0aGUgQmF5IG9mIEJlbmdhbCBhcyB0d28gZG9taW5hbnQgZHJpdmVycyBhZHZlcnNlbHkgaW1wYWN0aW5nIHdhdGVyIHF1YWxpdHkgYW5kIGluY3JlYXNpbmcgdnVsbmVyYWJpbGl0aWVzIG9mIHRoZSBlbnRpcmUgc21hbGwtc2NhbGUgZmlzaGVyaWVzIHNvY2lhbC1lY29sb2dpY2FsIHN5c3RlbS4gT3VyIGFuYWx5c2lzIHN1Z2dlc3RzIHRoYXQgdGhlcmUgYXJlIHN0cm9uZyBpbnRlcmNvbm5lY3Rpb25zIGJldHdlZW4gY2hhbmdlcyBpbiB3YXRlciBxdWFsaXR5IGFuZCB0aGUgbGV2ZWxzIG9mIHZ1bG5lcmFiaWxpdGllcyBpbiB0aGUgU1NGcyBvZiBDaGlsaWthIExhZ29vbi4gUG9sbHV0YW50cyBzdWNoIGFzIHBlc3RpY2lkZXMsIGFuZCBvcmdhbmljIGNvbXBvdW5kcyBhY2N1bXVsYXRlIGluIGZpc2ggdGlzc3VlcyBhbmQgYWZmZWN0IHRoZWlyIGdyb3d0aCwgcmVwcm9kdWN0aW9uLCBhbmQgb3ZlcmFsbCBoZWFsdGguIFRoaXMgbGVkIHRvIGRlY2xpbmVzIGluIGZpc2ggcG9wdWxhdGlvbnMsIG1ha2luZyBpdCBtb3JlIGRpZmZpY3VsdCBmb3IgZmlzaGVycyB0byBtYWtlIGEgbGl2aW5nLiBJbiBhZGRpdGlvbiB0byBkaXJlY3QgaW1wYWN0cyBvbiBmaXNoIHBvcHVsYXRpb25zLCBwb29yIHdhdGVyIHF1YWxpdHkgYWxzbyBoYXMgaW5kaXJlY3QgZWZmZWN0cyBvbiB0aGUgc29jaWFsIGFuZCBlY29ub21pYyB2dWxuZXJhYmlsaXR5IG9mIFNTRnMuIEZvciBleGFtcGxlLCBjb250YW1pbmF0aW9uIG9mIHdhdGVyIHNvdXJjZXMgbGVkIHRvIHRoZSByZWR1Y2VkIG51bWJlciBvZiBmaXNoIHNwZWNpZXMgcmVkdWNpbmcgdGhlIGFtb3VudCBvZiB0aW1lIGZpc2hlcnMgY2FuIHNwZW5kIG9uIGZpc2hpbmcgYWN0aXZpdGllcy4gVGhpcyBhbHNvIGFmZmVjdGVkIHRoZSBtYXJrZXRhYmlsaXR5IG9mIGZpc2ggcHJvZHVjdHMsIHJlZHVjZWQgaW5jb21lIGFuZCBpbmNyZWFzZWQgcG92ZXJ0eS4gVG8gZnVsbHkgdW5kZXJzdGFuZCB0aGUgaW50ZXJjb25uZWN0aW9ucyBiZXR3ZWVuIHdhdGVyIHF1YWxpdHkgYW5kIHZ1bG5lcmFiaWxpdHkgaW4gU1NGcyBpbiBDaGlsaWthIExhZ29vbiwgaXQgaXMgaW1wb3J0YW50IHRvIGNvbnNpZGVyIGJvdGggZW52aXJvbm1lbnRhbCBhbmQgc29jaWFsIGZhY3RvcnMsIGFzIHdlbGwgYXMgdGhlIGNvbXBsZXggZmVlZGJhY2sgbG9vcHMgYmV0d2VlbiB0aGVzZSBmYWN0b3JzLiBUaGUgc3R1ZHkgaGVscHMgaW4gYnJpZGdpbmcgYSBjcnVjaWFsIGdhcCBpbiBvdXIgdW5kZXJzdGFuZGluZyBvZiB0aGUgcm9sZSBvZiB3YXRlciBxdWFsaXR5IGluIHZ1bG5lcmFiaWxpdHkgYW5hbHlzaXMgd2l0aGluIHJlc291cmNlIGRlcGVuZGVudCBjb21tdW5pdGllcy4gV2UgY29uY2x1ZGUgd2l0aCBrZXkgaW5zaWdodHMgb24gcG9zc2libGUgY29waW5nIHJlc3BvbnNlcyBhbmQgYWRhcHRpdmUgY2FwYWNpdHkgbmVjZXNzYXJ5IGZvciB0aGUgc21hbGwtc2NhbGUgZmlzaGVyaWVzIGNvbW11bml0aWVzIHRvIHRyYW5zaXRpb24gdG93YXJkIHZpYWJpbGl0eS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMzM4OS9mbWFycy4yMDIzLjEwODcyOTYiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vd3d3LmZyb250aWVyc2luLm9yZy9hcnRpY2xlcy8xMC4zMzg5L2ZtYXJzLjIwMjMuMTA4NzI5Ni9wZGYiLCJpc3N1ZWREYXRlIjoiMjAyMy0wNi0yOSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZyb250aWVycyBpbiBNYXJpbmUgU2NpZW5jZSIsInZvbHVtZSI6MTAsImltcGFjdEZhY3RvciI6IjQuMjgifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRnJvbnRpZXJzIE1lZGlhIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo1MSwiY2l0ZWRCeUNvdW50Ijo1fX0sInF1b3RlIjoiV2F0ZXIgcXVhbGl0eSBvZiBDaGlsaWthIExhZ29vbiBoYXMgZGVncmFkZWQgZHVlIHRvIGFudGhyb3BvZ2VuaWMgaW50ZXJ2ZW50aW9ucyBhbmQgbmF0dXJhbCBmb3JjZXMgb2YgY2hhbmdlLiBWYXJpb3VzIGFjdGl2aXRpZXMgaW4gdGhlIENoaWxpa2EgTGFnb29uLCBzdWNoIGFzIGluZHVzdHJpYWwgd2FzdGV3YXRlciBkaXNwb3NhbCwgc2V3YWdlIGR1bXBpbmcsIGFxdWFjdWx0dXJlLCBoeWRyb2xvZ2ljYWwgaW50ZXJ2ZW50aW9ucywgYW5kIGN5Y2xvbmVzLCBhcmUgcHJvZHVjaW5nIHNhbGluaXR5IGNoYW5nZXMsIHNlZGltZW50IGRlcG9zaXRpb24sIG51dHJpZW50IGVucmljaG1lbnQsIGV1dHJvcGhpY2F0aW9uLCBhbmQgZGVhZCB6b25lcywgYXMgc3RhdGVkLiBGaXNoIHN1cnZpdmFsIGlzIGF0IHJpc2sgYXMgYSByZXN1bHQgb2YgdGhlIHNpZ25pZmljYW50IHJlZHVjdGlvbiBpbiBlbnZpcm9ubWVudGFsIGNpcmN1bXN0YW5jZXMsIHdoaWNoIGRpbWluaXNoZXMgZmlzaGVybWVu4oCZcyBpbmNvbWUgYW5kIHB1c2hlcyB0aGVtIG91dCBvZiBzb2NpZXR5LiBUaGVzZSBjYXVzZXMgYWZmZWN0aW5nIHRoZSB3YXRlciBxdWFsaXR5IG9mIHRoZSBMYWdvb24gZGVwaWN0IG1hbnkgZmFjZXMgb2YgdnVsbmVyYWJpbGl0eSBpbiBTU0YgY29tbXVuaXRpZXMuIEFzIHRoZSBDaGlsaWthIExhZ29vbiBlbnZpcm9ubWVudCBpcyBleHBlY3RlZCB0byB1bmRlcmdvIGluY3JlYXNpbmdseSBkcmFzdGljIGNoYW5nZXMgKHN1Y2ggYXMgY2F0Y2ggY29tcG9zaXRpb24sIGNhdGNoIGNhcGFjaXR5LCBiaW9kaXZlcnNpdHkgdmFyaWF0aW9ucywgd2F0ZXIgcXVhbGl0eSwgYW5kIGZpc2hlcnkgcmV2ZW51ZXMpLCBpbnRlcm5hdGlvbmFsIHBvbGljeSBoYXMgYmVjb21lIG1vcmUgaW50ZXJlc3RlZCBpbiBhZGFwdGF0aW9uIG1lYXN1cmVzLiBHaXZlbiBob3cgZGl2ZXJzZSBTU0YgZWNvc3lzdGVtcyBhcmUgZXhwZWN0ZWQgdG8gYmUgYmV0d2VlbiBjb250aW5lbnRzLCB0aGVyZSBpcyBubyDigJxvbmUgc2l6ZSBmaXRzIGFsbOKAnSBhcHByb2FjaC4gVGhlIHBhcGVyIG91dGxpbmVkIHNldmVyYWwgY29waW5nIGFuZCBhZGFwdGF0aW9uIG1lY2hhbmlzbXMgdG8gaW1wcm92ZSB3YXRlciBxdWFsaXR5IGFuZCBjb250aW51ZSBmaXNoaW5nIGluIGEgc3VzdGFpbmFibGUgd2F5LiBBZGFwdGF0aW9uIHN0cmF0ZWdpZXMgY2FuIG9jY2FzaW9uYWxseSBoYXZlIHVuZm9yZXNlZW4gY29uc2VxdWVuY2VzIGZvciBmaXNoZXJpZXMgYW5kIGZpc2hpbmcgY29tbXVuaXRpZXMuIEFueSB1bmV4cGVjdGVkIGNvbnNlcXVlbmNlcyBvZiBhZGFwdGF0aW9uIGNhbiBiZSBtaXRpZ2F0ZWQgdGhyb3VnaCBsb25nLXRlcm0gcGxhbm5pbmcgYW5kIGlkZW50aWZpY2F0aW9uIG9mIGZ1dHVyZSBvcHRpb25zIHRocm91Z2ggc2NlbmFyaW8gYW5hbHlzaXMuIFRoZSBmaW5kaW5ncyBvZiB0aGlzIHBhcGVyIGNhbiBiZSB1dGlsaXplZCB0byBkZXZlbG9wIHBvbGljaWVzIHRvIHByb3RlY3Qgd2F0ZXIgcXVhbGl0eSBhbmQgU1NGcyBpbiBhIHZhcmlldHkgb2YgY29hc3RhbCBsYWdvb24gc2V0dGluZ3MuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjIyLjI4NTAzNjA4NzAzNjEzMywibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ5MzA1MzQzNjI3OTMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjIzLjc3MzUxOTUxNTk5MTIxLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDgzMzY0MTA1MjI0NjEsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjI1LjI1NTU4NDcxNjc5Njg3NSwibGVmdCI6OC4zMzE1OTM1MTM0ODg3Nywid2lkdGgiOjQwLjQ4OTU1OTE3MzU4Mzk4NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjYuNzQ0MDY4MTQ1NzUxOTUzLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNTA2ODA1NDE5OTIxODc1LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjoyOC4yMjYxMzMzNDY1NTc2MTcsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40OTA0NjMyNTY4MzU5NCwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MjkuNzE0NjE2Nzc1NTEyNjk1LCJsZWZ0Ijo4LjMzMTU5MjU1OTgxNDQ1Mywid2lkdGgiOjQwLjQ5NDA5ODY2MzMzMDA4LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozMS4xOTY2ODM4ODM2NjY5OTIsImxlZnQiOjguMzMxNTkzNTEzNDg4NzcsIndpZHRoIjo0MC40OTQzOTIzOTUwMTk1MywiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6MzIuNjg1MTY1NDA1MjczNDQsImxlZnQiOjguMzMxNTk2Mzc0NTExNzE5LCJ3aWR0aCI6NDAuNDg3NDQ1ODMxMjk4ODMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM0LjE2NzIzMjUxMzQyNzczNCwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40OTc3NDU1MTM5MTYwMTYsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM1LjY1NTcxNTk0MjM4MjgxLCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ5ODYzODE1MzA3NjE3LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjozNy4xMzc3ODMwNTA1MzcxMSwibGVmdCI6MTAuNjg1MTg2Mzg2MTA4Mzk4LCJ3aWR0aCI6MzguMTQxMTA1NjUxODU1NDcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjM4LjYyNjI2NjQ3OTQ5MjE5LCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4NzE1MjA5OTYwOTM3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDAuMTA4MzMzNTg3NjQ2NDg0LCJsZWZ0Ijo4LjMzMzExMTc2MzAwMDQ4OCwid2lkdGgiOjQwLjQ4OTg1NjcxOTk3MDcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQxLjU5NjgxMzIwMTkwNDMsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDg4NjY2NTM0NDIzODMsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQzLjA3ODg4MDMxMDA1ODU5NCwibGVmdCI6OC4zMzMxMTE3NjMwMDA0ODgsIndpZHRoIjo0MC40ODE4NjExMTQ1MDE5NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDQuNTY3MzYzNzM5MDEzNjcsImxlZnQiOjguMzMzMTExNzYzMDAwNDg4LCJ3aWR0aCI6NDAuNDgzMDU4OTI5NDQzMzYsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjQ2LjA0OTQzMDg0NzE2Nzk3LCJsZWZ0Ijo4LjMzMzEwODkwMTk3NzUzOSwid2lkdGgiOjQwLjUwMDc0NzY4MDY2NDA2LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo0Ny41Mzc5MTQyNzYxMjMwNSwibGVmdCI6OC4zMzMxMDg5MDE5Nzc1MzksIndpZHRoIjo0MC40OTU2MDU0Njg3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NDkuMDE5OTgxMzg0Mjc3MzQ0LCJsZWZ0Ijo4LjMzMzEwNjA0MDk1NDU5LCJ3aWR0aCI6NDAuNTA2ODE2ODY0MDEzNjcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUwLjUwODQ2NDgxMzIzMjQyLCJsZWZ0Ijo4LjMzMzEwNjA0MDk1NDU5LCJ3aWR0aCI6NDAuNDg4NTA2MzE3MTM4NjcsImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUxLjk5MDUyODEwNjY4OTQ1LCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDk3NzI2NDQwNDI5NjksImhlaWdodCI6MS4xODA1MjA0MTUzMDYwOTEzfX0seyJwYWdlTnVtYmVyIjoxMiwiYmJveCI6eyJ0b3AiOjUzLjQ3OTAxMTUzNTY0NDUzLCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6MzMuMTU5NDQ2NzE2MzA4NTk0LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319LHsicGFnZU51bWJlciI6MTIsImJib3giOnsidG9wIjo1NC45NjEwNzg2NDM3OTg4MywibGVmdCI6MTAuNjgzNjcwMDQzOTQ1MzEyLCJ3aWR0aCI6MzguMTM2NTk2Njc5Njg3NSwiaGVpZ2h0IjoxLjE4MDUyMDQxNTMwNjA5MTN9fSx7InBhZ2VOdW1iZXIiOjEyLCJiYm94Ijp7InRvcCI6NTYuNDQ5NTYyMDcyNzUzOTA2LCJsZWZ0Ijo4LjMzMTU5MzUxMzQ4ODc3LCJ3aWR0aCI6NDAuNDg3MTQ0NDcwMjE0ODQ0LCJoZWlnaHQiOjEuMTgwNTIwNDE1MzA2MDkxM319XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEzIn19XQ%3D%3D. Smartphone-monitored sensors can track turbidity drops, crowdsourcing data via platforms like DataStream for adaptive scaling (Simon et al., 2023)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkRhbmEgRi4iLCJmYW1pbHkiOiJTaW1vbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJHYWJyaWVsIiwiZmFtaWx5IjoiTXVub3oifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUXVvYyBUdWMiLCJmYW1pbHkiOiJEaW5oIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlN1bmcgVm8iLCJmYW1pbHkiOiJEdXkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiS2F0IiwiZmFtaWx5IjoiS2F2YW5hZ2gifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUm9iZXJ0IEouIiwiZmFtaWx5IjoiU21pdGgifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQmFycnkiLCJmYW1pbHkiOiJIdXNrIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlPDqWJhc3RpZW4iLCJmYW1pbHkiOiJTYXV2w6kifX1dLCJpZHMiOnsiaXNzbiI6WyIyMDU3LTQ5OTEiXSwiaXNibiI6W10sImRvaSI6IjEwLjUzMzQvY3N0cC42NTUiLCJvcGVuYWxleCI6Ilc0MzkwNDA1NzM0In0sInRpdGxlIjoiQWRvcHQgYSBMYWtlOiBTdWNjZXNzZnVsbHkgVHJhY2tpbmcgSGFybWZ1bCBDeWFub2JhY3RlcmlhbCBCbG9vbXMgaW4gQ2FuYWRpYW4gU3VyZmFjZSBXYXRlcnMgVGhyb3VnaCBDaXRpemVuIFNjaWVuY2UiLCJhYnN0cmFjdCI6IlRoZSBwcm9saWZlcmF0aW9uIG9mIGhhcm1mdWwgd2F0ZXJib3JuZSBjeWFub2JhY3RlcmlhbCBhbGdhbCBibG9vbXMsIHNvbWUgb2Ygd2hpY2ggY2FuIHByb2R1Y2UgcG90ZW50IHRveGlucywgcG9zZXMgc2V2ZXJlIHJpc2tzIHRvIGVudmlyb25tZW50YWwgYW5kIGh1bWFuIGhlYWx0aC4gQWNhZGVtaWMgYW5kIGdvdmVybm1lbnRhbCBtb25pdG9yaW5nIGVmZm9ydHMgbWF5IGJlIGNvbnN0cmFpbmVkIGJ5IGJ1ZGdldCwgdGltZSwgYW5kIHN0YWZmLCBhbmQgdGh1cyBtaXNzIG90aGVyd2lzZSBzaWduaWZpY2FudCBwb2xsdXRpb24gZXZlbnRzLiBIZXJlLCB3ZSByZXBvcnQgb24gdGhlIGltcGxlbWVudGF0aW9uIG9mIGEgY2l0aXplbiBzY2llbmNlIHByb2plY3QgdG8gdHJhY2sgaGFybWZ1bCBjeWFub2JhY3RlcmlhbCBibG9vbXMgaW4gbGFrZXMgYW5kIHdhdGVyd2F5cyBhY3Jvc3MgQ2FuYWRhLiBUaHJvdWdoIGJvdGggY3Jvd2Rzb3VyY2luZyBhbmQgY3Jvd2RmdW5kaW5nLCB0aGUgQWRvcHQgYSBMYWtlIChBZG9wdCBhIExha2UgMjAyMikgY2FtcGFpZ24gYWltZWQgdG8gZG9jdW1lbnQgdGhlIHBvdGVudGlhbCBwcmVzZW5jZSBvZiBjeWFub2JhY3RlcmlhIGFuZCB0b3hpbnMgd2l0aCB0aGUgYXNzaXN0YW5jZSBvZiBwYXJ0aWNpcGFudHMsIHRodXMgaW1wcm92aW5nIHB1YmxpYyBhd2FyZW5lc3Mgb2YgdGhlIGlzc3VlIG9mIHdhdGVyIHF1YWxpdHkgcHJlc2VydmF0aW9uLiBVc2luZyBzb2NpYWwgbWVkaWEsIHBhcnRpY2lwYW50cyB3ZXJlIGVuY291cmFnZWQgdG8gcGFydGljaXBhdGUgaW4gdGhlIGluaXRpYXRpdmUgYnkgY29sbGVjdGluZyBzYW1wbGVzIGR1cmluZyBhIGJsb29tIGZyb20gYSBuZWFyYnkgcG9uZCBvciBieSBtYWtpbmcgYSBmaW5hbmNpYWwgY29udHJpYnV0aW9uIHRvIHN1cHBvcnQgdGhlIGluaXRpYXRpdmUuIEFkb3B0IGEgTGFrZSBiZW5lZml0dGVkIGZyb20gdGhlIGFuYWx5dGljYWwgcGxhdGZvcm0gb2YgQWxnYWwgQmxvb21zIFRyZWF0bWVudCwgUmlzayBBc3Nlc3NtZW50LCBQcmVkaWN0aW9ucywgYW5kIFByZXZlbnRpb24gKEFUUkFQUCksIGEgcmVzZWFyY2ggcHJvamVjdCBmb2N1c2VkIG9uIHRoZSBwcmVkaWN0aW9uIGFuZCBtYW5hZ2VtZW50IG9mIGhhcm1mdWwgY3lhbm9iYWN0ZXJpYWwgYmxvb21zLiBUaGUgcHJlc2VuY2Ugb2YgY3lhbm90b3hpbnMsIHdoaWNoIGNhbiBjb25maXJtIHdoZXRoZXIgYSBsYWtlIGhhcyBhIHRveGljIGJsb29tLCB3YXMgZGV0ZXJtaW5lZCB0aHJvdWdoIGhpZ2gtcmVzb2x1dGlvbiBtYXNzIHNwZWN0cm9tZXRyeSBhbmFseXNlcy4gVGhpcyBwYXBlciBwcmVzZW50cyBhbiBvdmVydmlldyBvZiB0aGUgaW1wbGVtZW50YXRpb24gb2YgdGhlIEFkb3B0IGEgTGFrZSBpbml0aWF0aXZlLCB0aGUgY2FtcGFpZ27igJlzIHN0YXR1cywgYW5kIHRoZSBsZXNzb25zIGxlYXJuZWQsIGFuZCBpdCBhcmd1ZXMgdGhlIGltcG9ydGFuY2Ugb2YgY29udGludWFsIG1vbml0b3Jpbmcgb2YgY3lhbm9iYWN0ZXJpYWwgYmxvb21zLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC41MzM0L2NzdHAuNjU1IiwicmVzb3VyY2VVcmwiOiJodHRwczovL3N0b3JhZ2UuZ29vZ2xlYXBpcy5jb20vam5sLXVwLWotY3N0cC1maWxlcy9qb3VybmFscy8xL2FydGljbGVzLzY1NS82NThlOWQ3ZDQ2YTBjLnBkZiIsImlzc3VlZERhdGUiOiIyMDIzLTEyLTI5IiwiYWNjZXNzZWREYXRlIjoiMjAyNS0wOCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiQ2l0aXplbiBTY2llbmNlIFRoZW9yeSBhbmQgUHJhY3RpY2UiLCJpc3N1ZSI6MSwidm9sdW1lIjo4LCJpbXBhY3RGYWN0b3IiOiIzLjEyIn0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6NjYsImxhc3RQYWdlIjo2Nn19LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJVYmlxdWl0eSBQcmVzcyJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MjcsImNpdGVkQnlDb3VudCI6Mn19LCJxdW90ZSI6IkluIHRoZSBVbml0ZWQgIFN0YXRlcywgYSB2b2x1bnRlZXIgbW9uaXRvcmluZyBwcm9ncmFtIGRldmVsb3BlZCBieSB0aGUgIFdhYmFzaCBSaXZlciBFbmhhbmNlbWVudCBDb3Jwb3JhdGlvbiBoZWxwZWQgdG8gaWRlbnRpZnkgIHdhdGVyLXF1YWxpdHkgcHJpb3JpdGllcyBhY3Jvc3MgdGhlIHdhdGVyc2hlZCAoTXVlbmljaCAgZXQgYWwuIDIwMTYpLiBJbiBDYW5hZGEsIG9yZ2FuaXphdGlvbnMgYXJlIGNvbW1pdHRlZCAgdG8gbW9uaXRvciBhbmQgcHJvdGVjdCB3YXRlciB0aHJvdWdoIGNyb3dkc291cmNlZCAgbW9uaXRvcmluZyBhbmQga25vd2xlZGdlIHRyYW5zZmVyIGluaXRpYXRpdmVzIChHM0UgIDIwMjI7IE90dGF3YSBSaXZlcmtlZXBlciAyMDIzOyBPdXIgTGl2aW5nIFdhdGVycyAyMDIyOyAgV2F0ZXIgUmFuZ2VycyAyMDIyOyBaaGFvLCBMaXUgYW5kIFdlaSAyMDIwKS5UaGVzZSAgZWZmb3J0cyBhcmUgY29tcGxlbWVudGVkIGJ5IG9wZW4gZnJlZSBwbGF0Zm9ybXMgc3VjaCAgYXMgRGF0YVN0cmVhbSwgd2hlcmUgd2F0ZXIgcXVhbGl0eSBkYXRhIGNvbGxlY3RlZCBmcm9tICB2YXJpb3VzIGNvbW11bml0eS1nYXRoZXJlZCBwcm9ncmFtcyBpcyBjdXJhdGVkLCBzdG9yZWQsICBhbmQgc2hhcmVkIHdpdGggdGhlIHB1YmxpYyAoVGhlIEdvcmRvbiBGb3VuZGF0aW9uIDIwMjMpLiBHaXZlbiB0aGUgY3JpdGljYWwgaW1wb3J0YW5jZSBvZiB3YXRlciByZXNvdXJjZXMgYW5kIHRoZWlyIGRpdmVyc2UgdXNlcywgaW5jbHVkaW5nIGRvbWVzdGljIHdlbGxzLCBtdW5pY2lwYWwgZHJpbmtpbmcgd2F0ZXIgcHJvZHVjdGlvbiwgZmlzaGluZywgcmVjcmVhdGlvbmFsIGFjdGl2aXRpZXMsICBhbmQgY3JvcCBpcnJpZ2F0aW9uLCBpdCBpcyBlc3NlbnRpYWwgdG8gcHJvdGVjdCBhbmQgbW9uaXRvciAgdGhlbS4gTGFrZXMsIG9mdGVuIHVzZWQgZm9yIHJlY3JlYXRpb25hbCBhY3Rpdml0aWVzIGFuZCBhcyAgZHJpbmtpbmcgd2F0ZXIgc291cmNlcywgYXJlIHN1c2NlcHRpYmxlIHRvIGJsb29tcyBvZiBhbGdhZSAgYW5kIGN5YW5vYmFjdGVyaWEsIHBhcnRpY3VsYXJseSBpbiB3YXJtLCBzbG93LW1vdmluZywgYW5kICBudXRyaWVudC1yaWNoIHdhdGVycy4gVGhlc2UgYmxvb21zIGNhbiBtYW5pZmVzdCBhcyBmb2FtLCAgc2N1bSwgbWF0cywgb3IgcGFpbnQtbGlrZSBjb2F0aW5ncyBvbiB0aGUgd2F0ZXLigJlzIHN1cmZhY2UgIGFuZCBjYW4gcHJvZHVjZSBjeWFub3RveGlucyB0aGF0IHBvc2UgaGVhbHRoIHJpc2tzIHRvICBodW1hbnMgYW5kIHdpbGRsaWZlIChGaWd1cmUgMSkuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MTIuNDY0MDcyMjI3NDc4MDI3LCJsZWZ0Ijo1MS44NTQ4ODg5MTYwMTU2MjUsIndpZHRoIjozOC43ODY3NTQ2MDgxNTQzLCJoZWlnaHQiOjEuMzU2ODE4NDM3NTc2Mjk0fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MTQuMTA0OTc4NTYxNDAxMzY3LCJsZWZ0Ijo1MS44NTM0MTY0NDI4NzEwOTQsIndpZHRoIjozOC43ODk2OTE5MjUwNDg4MywiaGVpZ2h0IjoxLjM1NjgxODQzNzU3NjI5NH19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjE1Ljc0NTg4Mzk0MTY1MDM5LCJsZWZ0Ijo1MS44NTM0MTY0NDI4NzEwOTQsIndpZHRoIjozOC43ODY3MzkzNDkzNjUyMzQsImhlaWdodCI6MS4zNTY4MTg0Mzc1NzYyOTR9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoxNy4zODY3ODkzMjE4OTk0MTQsImxlZnQiOjUxLjg1MzQxNjQ0Mjg3MTA5NCwid2lkdGgiOjM4Ljc4ODIxNTYzNzIwNzAzLCJoZWlnaHQiOjEuMzU2ODE4NDM3NTc2Mjk0fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MTkuMDI3Njk0NzAyMTQ4NDM4LCJsZWZ0Ijo1MS44NTM0MTY0NDI4NzEwOTQsIndpZHRoIjozOC43ODgyMTU2MzcyMDcwMywiaGVpZ2h0IjoxLjM1NjgxODQzNzU3NjI5NH19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjIwLjY2ODYwMDA4MjM5NzQ2LCJsZWZ0Ijo1MS44NTM0MTY0NDI4NzEwOTQsIndpZHRoIjozOC43ODUyNzQ1MDU2MTUyMzQsImhlaWdodCI6MS4zNTY4MTg0Mzc1NzYyOTR9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyMi4zMDk1MDU0NjI2NDY0ODQsImxlZnQiOjUxLjg1MzQxNjQ0Mjg3MTA5NCwid2lkdGgiOjM4Ljc4ODIzNDcxMDY5MzM2LCJoZWlnaHQiOjEuMzU2ODE4NDM3NTc2Mjk0fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MjMuOTUwNDEwODQyODk1NTA4LCJsZWZ0Ijo1MS44NTM0MTI2MjgxNzM4Mywid2lkdGgiOjM4Ljc4MzgzMjU1MDA0ODgzLCJoZWlnaHQiOjEuMzU2ODE4NDM3NTc2Mjk0fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MjUuNTkxMzE2MjIzMTQ0NTMsImxlZnQiOjUxLjg0OTAwNjY1MjgzMjAzLCJ3aWR0aCI6MzguNzkxMTcyMDI3NTg3ODksImhlaWdodCI6MS4zNTY4MTg0Mzc1NzYyOTR9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyNy4yMzIyMjE2MDMzOTM1NTUsImxlZnQiOjUxLjg1MTk0MDE1NTAyOTMsIndpZHRoIjozOC43OTI2MTc3OTc4NTE1NiwiaGVpZ2h0IjoxLjM1NjgxODQzNzU3NjI5NH19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjI4Ljg3MzEyNjk4MzY0MjU3OCwibGVmdCI6NTEuODUxOTQwMTU1MDI5Mywid2lkdGgiOjM4Ljc4Mzc3NTMyOTU4OTg0NCwiaGVpZ2h0IjoxLjM1NjgxODQzNzU3NjI5NH19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMwLjUxNDAzNDI3MTI0MDIzNCwibGVmdCI6NTEuODUxOTQwMTU1MDI5Mywid2lkdGgiOjM4Ljc4ODIzMDg5NTk5NjA5NCwiaGVpZ2h0IjoxLjM1NjgxODQzNzU3NjI5NH19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMyLjE1NDkzNzc0NDE0MDYyNSwibGVmdCI6NTEuODUxOTQwMTU1MDI5Mywid2lkdGgiOjM4LjU4NTI4OTAwMTQ2NDg0NCwiaGVpZ2h0IjoxLjM1NjgxODQzNzU3NjI5NH19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMzLjc5NTg0NTAzMTczODI4LCJsZWZ0Ijo1My43MDQ4Nzk3NjA3NDIxOSwid2lkdGgiOjM2LjkzNTI3OTg0NjE5MTQwNiwiaGVpZ2h0IjoxLjM1NjgxODQzNzU3NjI5NH19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjM1LjQzNjc1MjMxOTMzNTk0LCJsZWZ0Ijo1MS44NTE5NDAxNTUwMjkzLCJ3aWR0aCI6MzguNzg5NzA3MTgzODM3ODksImhlaWdodCI6MS4zNTY4MTg0Mzc1NzYyOTR9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNy4wNzc2NjM0MjE2MzA4NiwibGVmdCI6NTEuODUxOTQwMTU1MDI5Mywid2lkdGgiOjM4Ljc4Njc2NjA1MjI0NjA5NCwiaGVpZ2h0IjoxLjM1NjgxODQzNzU3NjI5NH19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjM4LjcxODU3MDcwOTIyODUxNiwibGVmdCI6NTEuODUxOTQwMTU1MDI5Mywid2lkdGgiOjM4Ljc4MzgxMzQ3NjU2MjUsImhlaWdodCI6MS4zNTY4MTg0Mzc1NzYyOTR9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0MC4zNTk0ODE4MTE1MjM0NCwibGVmdCI6NTEuODUxOTQwMTU1MDI5Mywid2lkdGgiOjM4Ljc4Njc3MzY4MTY0MDYyNSwiaGVpZ2h0IjoxLjM1NjgxODQzNzU3NjI5NH19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjQyLjAwMDM5MjkxMzgxODM2LCJsZWZ0Ijo1MS44NTE5NDAxNTUwMjkzLCJ3aWR0aCI6MzguNzg1MzAxMjA4NDk2MDk0LCJoZWlnaHQiOjEuMzU2ODE4NDM3NTc2Mjk0fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDMuNjQxMzAwMjAxNDE2MDE2LCJsZWZ0Ijo1MS44NTE5NDAxNTUwMjkzLCJ3aWR0aCI6MzguNzkxMTUyOTU0MTAxNTYsImhlaWdodCI6MS4zNTY4MTg0Mzc1NzYyOTR9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0NS4yODIyMDc0ODkwMTM2NywibGVmdCI6NTEuODUxOTQwMTU1MDI5Mywid2lkdGgiOjM4Ljc4OTcwNzE4MzgzNzg5LCJoZWlnaHQiOjEuMzU2ODE4NDM3NTc2Mjk0fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDYuOTIzMTE4NTkxMzA4NTk0LCJsZWZ0Ijo1MS44NTE5NDAxNTUwMjkzLCJ3aWR0aCI6MzguNzkyNjI5MjQxOTQzMzYsImhlaWdodCI6MS4zNTY4MTg0Mzc1NzYyOTR9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0OC41NjQwMjk2OTM2MDM1MTYsImxlZnQiOjUxLjg1MTk0MDE1NTAyOTMsIndpZHRoIjozOC43ODUyNzgzMjAzMTI1LCJoZWlnaHQiOjEuMzU2ODE4NDM3NTc2Mjk0fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTAuMjA0OTM2OTgxMjAxMTcsImxlZnQiOjUxLjg1MTk0MDE1NTAyOTMsIndpZHRoIjozOC43ODY3NjYwNTIyNDYwOTQsImhlaWdodCI6MS4zNTY4MTg0Mzc1NzYyOTR9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiI2NyJ9fV0%3D.

Riparian buffer zones vegetated strips along lake shores intercept non-point source runoff, trapping sediments and adsorbing pollutants before they enter water bodies. In villages around Chilika Lagoon or Rewalsar Lake, communities plant mixed buffers of vetiver grass, neem trees, and bamboo, creating 10-20m wide green belts that stabilize banks eroded by monsoons (Kumar & Mahajan, 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlBhd2FuIiwiZmFtaWx5IjoiS3VtYXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW1icmlzaCBLdW1hciIsImZhbWlseSI6Ik1haGFqYW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIyNTIzLTM5NjMiLCIyNTIzLTM5NzEiXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMDcvczQyNDUyLTAyMC0zMDgyLTgiLCJtYWciOiIzMDM1OTYwMjI4Iiwib3BlbmFsZXgiOiJXMzAzNTk2MDIyOCJ9LCJ0aXRsZSI6IlRyb3BoaWMgc3RhdHVzIGFuZCBpdHMgcmVndWxhdGluZyBmYWN0b3IgZGV0ZXJtaW5hdGlvbiBhdCB0aGUgUmV3YWxzYXIgTGFrZSwgbm9ydGh3ZXN0IEhpbWFsYXlhIChIUCksIEluZGlhLCBiYXNlZCBvbiBzZWxlY3RlZCBwYXJhbWV0ZXJzIGFuZCBtdWx0aXZhcmlhdGUgc3RhdGlzdGljYWwgYW5hbHlzaXMiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAwNy9zNDI0NTItMDIwLTMwODItOCIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9saW5rLnNwcmluZ2VyLmNvbS9jb250ZW50L3BkZi8xMC4xMDA3L3M0MjQ1Mi0wMjAtMzA4Mi04LnBkZiIsImlzc3VlZERhdGUiOiIyMDIwLTA2LTIyIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiU04gQXBwbGllZCBTY2llbmNlcyIsImlzc3VlIjo3LCJ2b2x1bWUiOjIsImltcGFjdEZhY3RvciI6IjcuMzUifSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiU3ByaW5nZXIgTmF0dXJlIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo0MCwiY2l0ZWRCeUNvdW50IjoxOX19LCJxdW90ZSI6IkNoYW5naW5nIHF1YWxpdHkgaGFzIGNyZWF0ZWQgdGhlIHdhdGVyIHNjYXJjaXR5IGFuZCAgYSBodWdlIG1hc3Mgb2YgcG9wdWxhdGlvbiB0aHVzIGJlZW4gZGVwcml2ZWQgZnJvbSB1c2luZyAgdGhlIGZyZXNod2F0ZXIgZm9yIHZhcmlvdXMgcHVycG9zZXMgWzMsIDRdLiBIb3dldmVyLCB0aGUgIG9ibm94aW91cyBtYXRlcmlhbHMgdGhhdCBzdGF5IGluIHdhdGVyIGJvZGllcyBmb3IgYSBsb25nLXRpbWUgcGVyaW9kIGJyaW5nIGNoYW5nZXMgaW4gdGhlIHBoeXNpb2NoZW1pY2FsIGNoYXItYWN0ZXJpc3RpY3MgYXMgd2VsbCBhcyB0aGUgbmF0dXJhbCBzY2VuYXJpbyBvZiB0aGUgbGFrZXMgWzVdLiAgVGhlIEhpbWFsYXlhbiBsYWtlcyBhcmUgdW5kZXIgc3RyZXNzIGR1ZSB0byBtaXNtYW5hZ2VkIGh1bWFuIGFjdGl2aXRpZXMsIHdoaWNoIGluY3JlYXNlIG51dHJpZW50cyBhbmQgcG9sbHV0aW9uIGluIHRlcm1zIG9mIGhlYXZ5IG1ldGFscyB0b3hpY2l0eSBhbmQgYmFjdGVyaWFsIGNvbnRhbWluYXRpb24gIFs24oCTOF0uIFRoZSBtYW5hZ2VtZW50IGFuZCBwcmVzZXJ2YXRpb24gb2YgdGhlIGxha2VzIGF0ICBwcmVzZW50IGhhdmUgYmVjb21lIGEgZ3JlYXQgY2hhbGxlbmdlIGZvciBlbnZpcm9ubWVudGFsLWlzdHMgYW5kIG1hbmFnZXJzIFs1LCA5XS4gVGhlIGV1dHJvcGhpY2F0aW9uIGlzIGFsc28gYSBtYWluIHByb2JsZW0gb2YgdGhlICBmcmVzaHdhdGVyIGxha2VzLCBhbmQgaXQgaXMgc3BlY2lmaWVkIGZvciB0aGUgbGFrZXMgaGF2aW5nICBudXRyaWVudCBlbnJpY2htZW50LiBJdCBjYXVzZXMgZXhjZXNzaXZlIGdyb3d0aCBvZiBhbGdhZSAgYW5kIGlzIGNvbnNpZGVyZWQgdG8gYmUgcmVzcG9uc2libGUgZm9yIGRlZ3JhZGluZyB0aGUgIHF1YWxpdGF0aXZlIG5hdHVyZSBvZiB0aGUgbGFrZXMgWzUsIDEwXS4gVGhlIGV1dHJvcGhpY2F0aW9uICBpcyBmb2xsb3dlZCBieSB0aGUgYWxnYWwgYmxvb20sIHdoaWNoIGdlbmVyYXRlcyB0b3hpbiwgIHdvcnNlbnMgd2F0ZXIgcXVhbGl0eSwgZGVwbGV0ZXMgZGlzc29sdmVkIG94eWdlbiAoRE8pICogUGF3YW4gS3VtYXIsIHBhd2FuLmV2c0BnbWFpbC5jb20gfCAxRGVwYXJ0bWVudCBvZiBFbnZpcm9ubWVudGFsIFNjaWVuY2VzLCBDZW50cmFsIFVuaXZlcnNpdHkgb2YgSGltYWNoYWwgUHJhZGVzaCwgRGhhcmFtc2hhbGEgMTc2MjE1LCBJbmRpYS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2OC41Nzg0NjA2OTMzNTkzOCwibGVmdCI6OC41NzE0MTg3NjIyMDcwMzEsIndpZHRoIjo0MC4zMjc1MTQ2NDg0Mzc1LCJoZWlnaHQiOjEuNTE5ODU1MjYwODQ4OTk5fX0seyJwYWdlTnVtYmVyIjowLCJiYm94Ijp7InRvcCI6NzAuMTU4OTk2NTgyMDMxMjUsImxlZnQiOjguNTY5NzM4Mzg4MDYxNTIzLCJ3aWR0aCI6NDAuMzU0NzI0ODg0MDMzMiwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjcxLjczOTU0NzcyOTQ5MjE5LCJsZWZ0Ijo4LjU2OTczODM4ODA2MTUyMywid2lkdGgiOjQwLjM0OTY5MzI5ODMzOTg0NCwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjczLjMyMDA5MTI0NzU1ODYsImxlZnQiOjguNTY4MDU5OTIxMjY0NjQ4LCJ3aWR0aCI6NDAuMDIwMDg4MTk1ODAwNzgsImhlaWdodCI6MS41MTk4NTUyNjA4NDg5OTl9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo3NC45MDA2NDIzOTUwMTk1MywibGVmdCI6OC41NjgwNTk5MjEyNjQ2NDgsIndpZHRoIjozOS45OTk5Mjc1MjA3NTE5NSwiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjc2LjQ4MTE4NTkxMzA4NTk0LCJsZWZ0Ijo4LjU2ODA1NzA2MDI0MTcsIndpZHRoIjo0MC4zNTcwNzQ3Mzc1NDg4MywiaGVpZ2h0IjoxLjUxOTg1NTI2MDg0ODk5OX19LHsicGFnZU51bWJlciI6MCwiYmJveCI6eyJ0b3AiOjc4LjA2MTcyOTQzMTE1MjM0LCJsZWZ0Ijo4LjU2OTczNjQ4MDcxMjg5LCJ3aWR0aCI6NDAuMzUzNzEzOTg5MjU3ODEsImhlaWdodCI6MS41MTk4NTUyNjA4NDg5OTl9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo2MC42NzU3Mjc4NDQyMzgyOCwibGVmdCI6NTEuNDI3MTY1OTg1MTA3NDIsIndpZHRoIjo0MC4zNzg1NjY3NDE5NDMzNiwiaGVpZ2h0IjoxOC45MDU4NTUxNzg4MzMwMDh9fSx7InBhZ2VOdW1iZXIiOjAsImJib3giOnsidG9wIjo4MS45MjczMzc2NDY0ODQzOCwibGVmdCI6OC40MDE0OTc4NDA4ODEzNDgsIndpZHRoIjo3My45NTQ2OTY2NTUyNzM0NCwiaGVpZ2h0IjoyLjY3ODc2MjY3NDMzMTY2NX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEifX1d. These zones foster soil microbiomes that denitrify nitrates, while root systems enhance infiltration, diluting urban surfactants fueling HABs in Bellandur (Gobler et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkNocmlzdG9waGVyIEouIiwiZmFtaWx5IjoiR29ibGVyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlJ1dGggVy4iLCJmYW1pbHkiOiJEcmlua3dhdGVyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkEuIFcuIiwiZmFtaWx5IjoiQW50aG9ueSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJKZW5uaWZlciBBLiIsImZhbWlseSI6IkdvbGVza2kifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW5uIE1hcmllIEUuIiwiZmFtaWx5IjoiRmFtdWxhcm8tUGVjb3JhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1pY2hhZWwgQi4iLCJmYW1pbHkiOiJXYWxsYWNlIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik5vcmEgUi4gVy4iLCJmYW1pbHkiOiJTdHJhcXVhZGluZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJSb25vam95IiwiZmFtaWx5IjoiSGVtIn19XSwiaWRzIjp7Imlzc24iOlsiMTY2NC0zMDJYIl0sImlzYm4iOltdLCJkb2kiOiIxMC4zMzg5L2ZtaWNiLjIwMjQuMTQ2NDY4NiIsInBtaWQiOiIzOTU2ODk5NyIsIm9wZW5hbGV4IjoiVzQ0MDQxMjA5NTIiLCJleGEiOiJodHRwczovL3d3dy5mcm9udGllcnNpbi5vcmcvam91cm5hbHMvbWljcm9iaW9sb2d5L2FydGljbGVzLzEwLjMzODkvZm1pY2IuMjAyNC4xNDY0Njg2L2Z1bGwifSwidGl0bGUiOiJTZXdhZ2UtYW5kIGZlcnRpbGl6ZXItZGVyaXZlZCBudXRyaWVudHMgYWx0ZXIgdGhlIGludGVuc2l0eSwgZGl2ZXJzaXR5LCBhbmQgdG94aWNpdHkgb2YgaGFybWZ1bCBjeWFub2JhY3RlcmlhbCBibG9vbXMgaW4gZXV0cm9waGljIGxha2VzIiwiYWJzdHJhY3QiOiJDeWFub2JhY3RlcmlhbCBoYXJtZnVsIGFsZ2FsIGJsb29tcyAoQ0hBQnMpIGFyZSBwcm9tb3RlZCBieSBleGNlc3NpdmUgbnV0cmllbnQgbG9hZGluZyBhbmQsIHdoaWxlIGZlcnRpbGl6ZXJzIGFuZCBzZXdhZ2UgYXJlIHRoZSBtb3N0IHByZXZhbGVudCBleHRlcm5hbCBudXRyaWVudCBzb3VyY2VzIGluIG1vc3Qgd2F0ZXJzaGVkcywgdGhlIGRpZmZlcmVudGlhbCBlZmZlY3RzIG9mIHRoZXNlIG51dHJpZW50IHNvdXJjZXMgb24gQ0hBQnMgYXJlIHVua25vd24uIEhlcmUsIHdlIHRyYWNrZWQgQ0hBQnMgYW5kIHBlcmZvcm1lZCBleHBlcmltZW50cyBpbiBmaXZlIGRpc3RpbmN0IGxha2VzIGFjcm9zcyB0aGUgTm9ydGhlcm4gVVMgaW5jbHVkaW5nIExha2UgRXJpZS4gRmVydGlsaXplcnMgd2l0aCBhbW1vbml1bSBhbmQgb3J0aG9waG9zcGhhdGUsIG1lbWJyYW5lICgwLjIgzrxtKS1maWx0ZXJlZCBzZXdhZ2UgKGRvbWluYXRlZCBieSByZWR1Y2VkIGZvcm1zIG9mIG5pdHJvZ2VuKSBzYW5kLWFuZCBtZW1icmFuZS1maWx0ZXJlZCBzZXdhZ2UgKGRvbWluYXRlZCBieSBuaXRyYXRlKSwgYW5kIGFuIGlub3JnYW5pYyBudXRyaWVudCBzb2x1dGlvbiBvZiBhbW1vbml1bSBhbmQgb3J0aG9waG9zcGhhdGUgd2VyZSB1c2VkIGFzIGV4cGVyaW1lbnRhbCBudXRyaWVudCBzb3VyY2VzIGZvciBDSEFCcyBhdCBOLWVxdWl2YWxlbnQsIGVudmlyb25tZW50YWxseSByZWFsaXN0aWMgY29uY2VudHJhdGlvbnMuIFBoeXRvcGxhbmt0b24gY29tbXVuaXRpZXMgd2VyZSBldmFsdWF0ZWQgZmx1b3JvbWV0cmljYWxseSwgbWljcm9zY29waWNhbGx5LCBhbmQgdmlhIGhpZ2ggdGhyb3VnaHB1dCBzZXF1ZW5jaW5nIG9mIHRoZSAxNlMgclJOQSBnZW5lLCBhbmQgbGV2ZWxzIG9mIG1pY3JvY3lzdGluIGFuZCB0aGUgzrQgMTUgTiBjb250ZW50IG9mIHBhcnRpY3VsYXRlIG9yZ2FuaWMgbml0cm9nZW4gKM60UE8gMTUgTikgd2VyZSBxdWFudGlmaWVkLiBGZXJ0aWxpemVyIGFuZCBib3RoIHNvdXJjZXMgb2Ygd2FzdGV3YXRlciBpbmNyZWFzZWQgdGhlIGFidW5kYW5jZSBvZiBjeWFub2JhY3RlcmlhIGluIGFsbCBleHBlcmltZW50cyBhY3Jvc3MgYWxsIGZpdmUgbGFrZXMgKCBwICZhbXA7bHQ7IDAuMDUgZm9yIGFsbCkgd2hlcmVhcyBlZmZlY3RzIG9uIGV1a2FyeW90aWMgcGh5dG9wbGFua3RvbiB3ZXJlIGxpbWl0ZWQuIFNhbmQtZmlsdGVyZWQgc2V3YWdlIGNvbnRhaW5lZCBsZXNzIFAsIG9yZ2FuaWMgbWF0dGVyLCBhbmQgYW1tb25pdW0gYnV0IG1vcmUgbml0cmF0ZSBhbmQgaGFkIGEgMjUlIGxlc3MgcG90ZW50IHN0aW11bGF0b3J5IGVmZmVjdCBvbiBjeWFub2JhY3RlcmlhIHRoYW4gbWVtYnJhbmUtZmlsdGVyZWQgc2V3YWdlLCBzdWdnZXN0aW5nIG5pdHJpZmljYXRpb24gbWF5IHBsYXkgYSByb2xlIGluIHJlZHVjaW5nIENIQUJzLiBGZXJ0aWxpemVyIGluY3JlYXNlZCBtaWNyb2N5c3RpbiBsZXZlbHMgYW5kIGRlY3JlYXNlZCB0aGUgzrRQTyAxNSBOIHdoZXJlYXMgd2FzdGV3YXRlciBpbmNyZWFzZWQgzrRQTyAxNSBOICggcCAmYW1wO2x0OyAwLjA1IGZvciBhbGwpLiBNaWNyb2N5c3RpcyB3YXMgdGhlIGdlbnVzIG1vc3QgY29uc2lzdGVudGx5IHByb21vdGVkIGJ5IG51dHJpZW50IHNvdXJjZXMgKCBwICZhbXA7bHQ7IDAuMDUgaW4gYWxsIGV4cGVyaW1lbnRzKSwgZm9sbG93ZWQgYnkgQ3lhbm9iaXVtICggcCAmYW1wO2x0OyAwLjA1IGluIDUwJSBvZiBleHBlcmltZW50cyksIHdpdGggaW5jcmVhc2VzIGluIE1pY3JvY3lzdGlzIGJpb21hc3MgY29uc2lzdGVudGx5IGVsaWNpdGVkIGJ5IG1lbWJyYW5lLWZpbHRlcmVkIHdhc3Rld2F0ZXIuIENvbGxlY3RpdmVseSwgcmVzdWx0cyBkZW1vbnN0cmF0ZSB0aGF0IGRpZmZlcmluZyB0eXBlcyBvZiBzZXdhZ2UgZGlzY2hhcmdlIGFuZCBmZXJ0aWxpemVycyBjYW4gcHJvbW90ZSBDSEFCIGludGVuc2l0eSBhbmQgdG94aWNpdHksIHdoaWxlIGNvbmN1cnJlbnRseSBhbHRlcmluZyBDSEFCIGRpdmVyc2l0eSBhbmQgzrRQTyAxNSBOLiBXaGlsZSBtZW1icmFuZS1maWx0ZXJlZCBzZXdhZ2UgY29uc2lzdGVudGx5IGZhdm9yZWQgTWljcm9jeXN0aXMgLCB0aGUgZGlzY2hhcmdlIG9mIHNld2FnZSB0aHJvdWdoIHNhbmRzIG11dGVkIGJsb29tIGludGVuc2l0eSBzdWdnZXN0aW5nIHNhbmQtYmVkcyBtYXkgcmVwcmVzZW50IGEgdG9vbCB0byByZW1vdmUga2V5IG51dHJpZW50cyBhbmQgcGFydGlhbGx5IG1pdGlnYXRlIENIQUJzLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4zMzg5L2ZtaWNiLjIwMjQuMTQ2NDY4NiIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cuZnJvbnRpZXJzaW4ub3JnL2pvdXJuYWxzL21pY3JvYmlvbG9neS9hcnRpY2xlcy8xMC4zMzg5L2ZtaWNiLjIwMjQuMTQ2NDY4Ni9wZGYiLCJpc3N1ZWREYXRlIjoiMjAyNC0xMS0wNiIsImFjY2Vzc2VkRGF0ZSI6IjIwMjYtMDIiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZyb250aWVycyBpbiBNaWNyb2Jpb2xvZ3kiLCJ2b2x1bWUiOjE1LCJpbXBhY3RGYWN0b3IiOiIzLjk5In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTQ2NDY4NiwibGFzdFBhZ2UiOjE0NjQ2ODZ9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiRnJvbnRpZXJzIE1lZGlhIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo3MywiY2l0ZWRCeUNvdW50Ijo5fX0sInF1b3RlIjoiZGVyaXZlZCBudXRyaWVudHMgYWx0ZXIgdGhlIGludGVuc2l0eSwgZGl2ZXJzaXR5LCBhbmQgdG94aWNpdHkgb2YgaGFybWZ1bCBjeWFub2JhY3RlcmlhbCBibG9vbXMgLi4uIEN5YW5vYmFjdGVyaWFsIGhhcm1mdWwgYWxnYWwgYmxvb21zIChDSEFCcykgYXJlIHByb21vdGVkIGJ5IGV4Y2Vzc2l2ZSBudXRyaWVudCBsb2FkaW5nIGFuZCwgd2hpbGUgZmVydGlsaXplcnMgYW5kIHNld2FnZSBhcmUgdGhlIG1vc3QgcHJldmFsZW50IGV4dGVybmFsIG51dHJpZW50IHNvdXJjZXMgaW4gbW9zdCB3YXRlcnNoZWRzLCB0aGUgZGlmZmVyZW50aWFsIGVmZmVjdHMgb2YgdGhlc2UgbnV0cmllbnQgLi4uIG9yZ2FuaWMgbml0cm9nZW4gKM60UE8xNU4pIHdlcmUgcXVhbnRpZmllZC4gRmVydGlsaXplciBhbmQgYm90aCBzb3VyY2VzIG9mIHdhc3Rld2F0ZXIgaW5jcmVhc2VkIHRoZSBhYnVuZGFuY2Ugb2YgLi4uIHBoeXRvcGxhbmt0b24gd2VyZSBsaW1pdGVkLiBTYW5kLWZpbHRlcmVkIHNld2FnZSBjb250YWluZWQgbGVzcyBQLCBvcmdhbmljIG1hdHRlciwgYW5kIGFtbW9uaXVtIGJ1dCBtb3JlIG5pdHJhdGUgYW5kIC4uLiBkaWZmZXJpbmcgdHlwZXMgb2Ygc2V3YWdlIGRpc2NoYXJnZSBhbmQgZmVydGlsaXplcnMgY2FuIHByb21vdGUgQ0hBQiBpbnRlbnNpdHkgYW5kIHRveGljaXR5LCB3aGlsZSBjb25jdXJyZW50bHkgLi4uIHNld2FnZSB0aHJvdWdoIHNhbmRzIG11dGVkIGJsb29tIGludGVuc2l0eSBzdWdnZXN0aW5nIHNhbmQtYmVkcyBtYXkgcmVwcmVzZW50IGEgdG9vbCB0byByZW1vdmUga2V5IG51dHJpZW50cyBhbmQgLi4uICggV2F0c29uIGV0IGFsLiwgMjAxNiA7IEJ1bGxlcmphaG4gZXQgYWwuLCAyMDE2IDsgSHVpc21hbiBldCBhbC4sIDIwMTggKSBhbmQgZXhjZXNzaXZlIG51dHJpZW50IGxvYWRpbmcgaXMgdGhlIHByb2Nlc3MgbW9zdCBjb21tb25seSBjaXRlZCBhcyB0aGUgZW52aXJvbm1lbnRhbCBmYWN0b3IgcHJvbW90aW5nIENIQUJzICgifV0%3D. Maintenance involves community pruning and mulching, yielding co-benefits like fodder and shade for livestock. Studies show buffers reduce nutrient loads by 40-70%, averting biodiversity loss and toxin risks (Khorasani & Zhu, 2021)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhhbWVkIiwiZmFtaWx5IjoiS2hvcmFzYW5pIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlpoZW5kdW8iLCJmYW1pbHkiOiJaaHUifX1dLCJpZHMiOnsiaXNzbiI6WyIwMDIyLTE2OTQiLCIxODc5LTI3MDciXSwiaXNibiI6W10sImRvaSI6IjEwLjEwMTYvai5qaHlkcm9sLjIwMjEuMTI2ODg2IiwibWFnIjoiMzE3NzA3MTI5OCIsIm9wZW5hbGV4IjoiVzMxNzcwNzEyOTgifSwidGl0bGUiOiJQaG9zcGhvcnVzIHJldGVudGlvbiBpbiBsYWtlczogQSBjcml0aWNhbCByZWFzc2Vzc21lbnQgb2YgaHlwb3RoZXNlcyBhbmQgc3RhdGljIG1vZGVscyIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC4xMDE2L2ouamh5ZHJvbC4yMDIxLjEyNjg4NiIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly9lYXJ0aGFyeGl2Lm9yZy9yZXBvc2l0b3J5L29iamVjdC8yNDgxL2Rvd25sb2FkLzUwNzQvIiwiaXNzdWVkRGF0ZSI6IjIwMjEtMDktMDIiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA4IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJKb3VybmFsIG9mIEh5ZHJvbG9neSIsInZvbHVtZSI6NjAzLCJpbXBhY3RGYWN0b3IiOiI3LjY0In0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTI2ODg2LCJsYXN0UGFnZSI6MTI2ODg2fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkVsc2V2aWVyIEJWIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50Ijo3NywiY2l0ZWRCeUNvdW50IjoxMn19LCJxdW90ZSI6Ikhvd2V2ZXIsIGh1bWFuIGFjdGl2aXRpZXMgaGF2ZSA0OSBhY2NlbGVyYXRlZCB0aGUgZXV0cm9waGljYXRpb24gcHJvY2VzcyBieSBpbmNyZWFzaW5nIHRoZSBudXRyaWVudHMgZGVsaXZlcnkgdG8gdGhlIGFxdWF0aWMgc3lzdGVtcyA1MCAoTWVrb25uZW4gYW5kIEhvZWtzdHJhLCAyMDE4KS4gVGh1cywgYW50aHJvcG9nZW5pYyBldXRyb3BoaWNhdGlvbiBpcyBvbmUgb2YgdGhlIG1vc3QgaW1wb3J0YW50IDUxIGVsZW1lbnRzIG9mIGZyZXNoIGFuZCBtYXJpbmUgd2F0ZXIgcXVhbGl0eSBkZXRlcmlvcmF0aW9uIChIdSBldCBhbC4sIDIwMTk7IFNtaXRoIGFuZCBTY2hpbmRsZXIsIDUyIDIwMDkpLiBPbmUgZGlyZWN0IGNvbnNlcXVlbmNlIG9mIGFudGhyb3BvZ2VuaWMgZXV0cm9waGljYXRpb24gY29tZXMgaW4gdGhlIGZvcm0gb2YgbWFzc2l2ZSBhbGdhbCA1MyBibG9vbXMgKEdyYW7DqWxpIGV0IGFsLiwgMjAwODsgSGVpc2xlciBldCBhbC4sIDIwMDgpLCB3aGljaCBhcmUgcHJlZGljdGVkIHRvIGJlIGludGVuc2lmaWVkIHVuZGVyIDU0IHdhcm1lciB3YXRlciB0ZW1wZXJhdHVyZXMgYXMgY2xpbWF0ZSBjaGFuZ2VzIChHb2JsZXIsIDIwMjA7IE11a3VuZGFuIGV0IGFsLiwgMjAyMCkuIDU1IEV1dHJvcGhpY2F0aW9uIGlzIGEg4oCcd2lja2Vk4oCdIHByb2JsZW0sIHdoaWNoIGlzIHRoZSBjb25zZXF1ZW5jZSBvZiB2YXJpb3VzIHByb2Nlc3NlcyB0aGF0IG9wZXJhdGUgNTYgY3VtdWxhdGl2ZWx5LiBDb25zaWRlcmluZyB0aGUgdW5pcXVlbmVzcyBvZiBlYWNoIGxha2UgYW5kIGl0cyBzdXJyb3VuZGluZyBhcmVhLCB0aGVyZSBpcyBubyBicm9hZGx5IDU3IGFwcGxpY2FibGUgc2V0IG9mIGJlc3QgbWFuYWdlbWVudCBwcmFjdGljZXMgdGhhdCBjYW4gYmUgYXBwbGllZCBpbiB3YXRlcnNoZWRzIHRvIG1pdGlnYXRlIDU4IHBob3NwaG9ydXMgbG9hZGluZyBhbmQgaXRzIGltcGFjdCBvbiBhbGwgbGFrZXMgKFRob3JudG9uIGV0IGFsLiwgMjAxMykuIEhlbmNlLCBldXRyb3BoaWNhdGlvbiA1OSBtYW5hZ2VtZW50IGFuZCBsYWtlIHJlc3RvcmF0aW9uIG5lZWQgaW50ZWdyYXRlZCBwbGFucyB0aGF0IGFyZSBub3Qgb25seSBzY2llbnRpZmljYWxseSB2YWxpZCBidXQgNjAgYWxzbyBzb2Npby1lY29ub21pY2FsbHkgc2F0aXNmeWluZyAoR2lic29uIGV0IGFsLiwgMjAwMCkuICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6MzYuMTY4NDM0MTQzMDY2NDA2LCJsZWZ0IjoxMS43Njg2MjgxMjA0MjIzNjMsIndpZHRoIjo3Ni45NjMzOTQxNjUwMzkwNiwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjM2LjcxODQzMzM4MDEyNjk1LCJsZWZ0Ijo2Ljk4MDM5MjQ1NjA1NDY4NzUsIndpZHRoIjoyLjI1MDk1NzQ4OTAxMzY3MiwiaGVpZ2h0IjoxLjM5Mzk0MDQ0ODc2MDk4NjN9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjozOS42NTMyODU5ODAyMjQ2MSwibGVmdCI6Ni45ODAzOTI0NTYwNTQ2ODc1LCJ3aWR0aCI6ODEuNzYxNDI4ODMzMDA3ODEsImhlaWdodCI6Mi4wMjEyMTE2MjQxNDU1MDh9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo0My4xMzgxMzQwMDI2ODU1NSwibGVmdCI6Ni45ODAzOTI0NTYwNTQ2ODc1LCJ3aWR0aCI6ODEuNzU1NTYxODI4NjEzMjgsImhlaWdodCI6Mi4wMjEyMTE2MjQxNDU1MDh9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo0Ni42MjI5ODIwMjUxNDY0ODQsImxlZnQiOjYuOTgwMzkyNDU2MDU0Njg3NSwid2lkdGgiOjgxLjc0MjgwNTQ4MDk1NzAzLCJoZWlnaHQiOjIuMDIxMjExNjI0MTQ1NTA4fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6NTAuMTA3ODMwMDQ3NjA3NDIsImxlZnQiOjYuOTgwMzkyNDU2MDU0Njg3NSwid2lkdGgiOjgxLjc1MjYxNjg4MjMyNDIyLCJoZWlnaHQiOjIuMDIxMjExNjI0MTQ1NTA4fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6NTMuNTk1MjAzMzk5NjU4MiwibGVmdCI6Ni45ODAzOTI0NTYwNTQ2ODc1LCJ3aWR0aCI6ODEuNzQ4NzAzMDAyOTI5NjksImhlaWdodCI6Mi4wMjEyMTE2MjQxNDU1MDh9fSx7InBhZ2VOdW1iZXIiOjIsImJib3giOnsidG9wIjo1Ny4wODAwNDc2MDc0MjE4NzUsImxlZnQiOjYuOTgwMzkyNDU2MDU0Njg3NSwid2lkdGgiOjczLjMxMDQ1NTMyMjI2NTYyLCJoZWlnaHQiOjIuMDIxMjExNjI0MTQ1NTA4fX0seyJwYWdlTnVtYmVyIjoyLCJiYm94Ijp7InRvcCI6NjEuNTY0OTAzMjU5Mjc3MzQ0LCJsZWZ0Ijo2Ljk4MDM5MjQ1NjA1NDY4NzUsIndpZHRoIjo4MS43NTc1MTQ5NTM2MTMyOCwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjY1LjA0OTc0MzY1MjM0Mzc1LCJsZWZ0Ijo2Ljk4MDM5MjQ1NjA1NDY4NzUsIndpZHRoIjo4MS43NTk0NzU3MDgwMDc4MSwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjY4LjUzNDU5MTY3NDgwNDY5LCJsZWZ0Ijo2Ljk4MDM5MjQ1NjA1NDY4NzUsIndpZHRoIjo4MS43MDM5MzM3MTU4MjAzMSwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjcyLjAxOTQ1NDk1NjA1NDY5LCJsZWZ0Ijo2Ljk4MDM5MjQ1NjA1NDY4NzUsIndpZHRoIjo4MS43NTA2NTYxMjc5Mjk2OSwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjc1LjUwODA4NzE1ODIwMzEyLCJsZWZ0Ijo2Ljk4MDM5MjQ1NjA1NDY4NzUsIndpZHRoIjo4MS43NTc1MTQ5NTM2MTMyOCwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19LHsicGFnZU51bWJlciI6MiwiYmJveCI6eyJ0b3AiOjc4Ljk5MjkzNTE4MDY2NDA2LCJsZWZ0IjoxMS43Njg2MjgxMjA0MjIzNjMsIndpZHRoIjo3Ni45NzMxOTc5MzcwMTE3MiwiaGVpZ2h0IjoyLjAyMTIxMTYyNDE0NTUwOH19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjEyNjg4OCJ9fV0%3D. Linking to sacred groves in TEK traditions, these buffers buffer against corruption-plagued dredging, promoting long-term hypoxia prevention (Ji et al., 2024)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkRhb2JpbiIsImZhbWlseSI6IkppIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllhbnhpbmciLCJmYW1pbHkiOiJIYW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTGlhbmdob25nIiwiZmFtaWx5IjoiTG9uZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJYaWFva2FuZyIsImZhbWlseSI6IlhpbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJIdWl5YW4iLCJmYW1pbHkiOiJYdSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTaXFpYW4iLCJmYW1pbHkiOiJRaXUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSmlhbmdodWFpIiwiZmFtaWx5IjoiTWVuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJYaW5neGluZyIsImZhbWlseSI6IlpoYW8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiWWFuYW4iLCJmYW1pbHkiOiJIdWFuZyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJEZWZ1IiwiZmFtaWx5IjoiTGl1In19XSwiaWRzIjp7Imlzc24iOlsiMjI5Ni03MDFYIl0sImlzYm4iOltdLCJkb2kiOiIxMC4zMzg5L2Zldm8uMjAyMy4xMjk3MDQ3Iiwib3BlbmFsZXgiOiJXNDM5MDYxMDY2OCJ9LCJ0aXRsZSI6Ikh5cG94aWEgYW5kIGl0cyBmZWVkYmFjayByZXNwb25zZSB0byBhbGdhbCBibG9vbXMgYW5kIENINCBlbWlzc2lvbnMgaW4gc3VidHJvcGljYWwgcmVzZXJ2b2lycyIsImFic3RyYWN0IjoiQW50aHJvcG9nZW5pYyBudXRyaWVudCBpbnB1dCBzdGltdWxhdGVzIHN1cmZhY2UgZXV0cm9waGljYXRpb24gYW5kIGFsZ2FsIGJsb29tcyBhbmQgdGh1cyBleGFjZXJiYXRlcyBveHlnZW4gZGVwbGV0aW9uIGFuZCBoeXBveGlhIGluIGFxdWF0aWMgZWNvc3lzdGVtcy4gSG93ZXZlciwgZXhpc3Rpbmcgc3R1ZGllcyBoYXZlIHBhaWQgbGl0dGxlIGF0dGVudGlvbiBvbiB0aGUgdW5kZXJzdGFuZGluZyBvZiB0aGUgZmVlZGJhY2sgcmVsYXRpb25zaGlwcyBhbW9uZyBveHlnZW4gY29uc3VtcHRpb24sIGFsZ2FsIHN1Y2Nlc3Npb24gYW5kIGdyZWVuaG91c2UgZ2FzIGVmZmVjdCBhZnRlciByaXZlciBkYW1taW5nLCBwYXJ0aWN1bGFybHkgaW4gVGhyZWUgR29yZ2VzIFJlc2Vydm9pci4gVGhpcyBmaWVsZCB3b3JrIGludmVzdGlnYXRlZCB0aGUgdGhlcm1hbCBzdHJhdGlmaWNhdGlvbiwgd2F0ZXIgcXVhbGl0eSBhbmQgcGh5dG9wbGFua3RvbiBpbiBmb3VyIHRyaWJ1dGFyaWVzIChQZW5neGkgQmF5IChQWEIpLCBEYW5pbmcgQmF5IChETkIpLCBTaGVubm9uZyBCYXkgKFNOQiksIGFuZCBYaWFuZ3hpIEJheSAoWFhCKSkgb2YgdGhlIFRocmVlIEdvcmdlcyBSZXNlcnZvaXIgKFRHUikgaW4gc3ByaW5nIChBcHJpbCAyNS0yOCwgMjAyMiksIGFuZCBleHBsb3JlZCB0aGUgZHJpdmVycyBvZiBoeXBveGlhIGFuZCBpdHMgZmVlZGJhY2sgcmVzcG9uc2UgdG8gYWxnYWwgYmxvb21zIGFuZCBtZXRoYW5lIChDSCA0ICkgZW1pc3Npb25zLiBUaGUgcmVzdWx0cyBzaG93ZWQgdGhhdCBkdXJpbmcgdGhlIG9ic2VydmF0aW9uIHBlcmlvZCwgRE8sIHRoZXJtYWwgc3RyYXRpZmljYXRpb24sIGFuZCBhbGdhbCBibG9vbXMgd2VyZSBtb3JlIHNldmVyZSBpbiBQWEIuIFRoZSBoaWdoIENoZW1pY2FsIE94eWdlbiBEZW1hbmQgKENPRCBNbiApICgxMS4yNyBtZ%2BKAokwgLTEgKSBhbmQgRGlzc29sdmVkIE9yZ2FuaWMgQ2FyYm9uIChET0MpIGNvbnRlbnQgKDcuNTYgbWfigKJMIC0xICkgaW4gUFhCIHdlcmUgdGhlIG1haW4gY2F1c2VzIG9mIHRoZSBwcm9ncmVzc2l2ZSBoeXBveGlhLiBGdXJ0aGVybW9yZSwgc3Ryb25nZXIgZGVuc2l0eSBzdHJhdGlmaWNhdGlvbiBpbiBQWEIgaW5oaWJpdGVkIHRoZSB2ZXJ0aWNhbCBzdXBwbHkgb2YgRE8gYW5kIGFjY2VsZXJhdGVkIGJvdHRvbSBoeXBveGlhLiBCdXQgdGhlIGRvd25zbG9wZSBkZW5zaXR5IGN1cnJlbnQgcmVwbGVuaXNoZWQgYm90dG9tIERPIGluIHRoZSB1cHBlciByZWFjaGVzIG9mIHRoZSBETkIsIFNOQiwgYW5kIFhYQi4gVGhlIENINCBjb25jZW50cmF0aW9uIHdhcyBnZW5lcmFsbHkgaGlnaGVyIGluIGh5cG94aWEgem9uZSwgZXNwZWNpYWxseSBpbiB3aGVyZSBjeWFub2JhY3RlcmlhIGFyZSB0aGUgZG9taW5hbnQgc3BlY2llcyAoZWcuLCBQWDAzIGFuZCBQWDA0KS4gRGFtIGNvbnN0cnVjdGlvbiBsZWQgdG8gdGhlIHN1Y2Nlc3Npb24gb2YgYWxnYWwtZG9taW5hbnQgc3BlY2llcywgZXhlcnRpbmcgYSBzaWduaWZpY2FudCBpbmZsdWVuY2Ugb24gdGhlIHJpdmVyIGNhcmJvbiBjeWNsZS4gVGhlIGV4cGFuc2lvbiBvZiBjeWFub2JhY3RlcmlhbCBibG9vbXMgYW5kIHRoZSBjb25zZXF1ZW5jZSBvZiBtZXRhYm9saWMgc3Vic3RyYXRlcyBpbmNyZWFzZSBhcmUgZ3JhZHVhbGx5IGluY3JlYXNpbmcgb3h5Z2VuIGRlcHJpdmF0aW9uLCBldmVuIGh5cG94aWEsIGFuZCBlbmhhbmNpbmcgQ0ggNCBwcm9kdWN0aW9uIGluIHJpdmVycy4gVGhlc2UgZmluZGluZ3Mgc3VnZ2VzdCB0aGF0IHRoZSBzdWNjZXNzaW9uIG9mIGFsZ2FsLWRvbWluYW50IHNwZWNpZXMgY2F1c2VkIGJ5IHJpdmVyIGRhbW1pbmcgaGFzIGEgcHJvbm91bmNlZCBwb3NpdGl2ZSBmZWVkYmFjayBlZmZlY3Qgb24gcmVzZXJ2b2lyIGh5cG94aWEgcHJvY2Vzc2VzIGFuZCBncmVlbmhvdXNlIGVmZmVjdC4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMzM4OS9mZXZvLjIwMjMuMTI5NzA0NyIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cuZnJvbnRpZXJzaW4ub3JnL2FydGljbGVzLzEwLjMzODkvZmV2by4yMDIzLjEyOTcwNDcvcGRmP2lzUHVibGlzaGVkVjI9RmFsc2UiLCJpc3N1ZWREYXRlIjoiMjAyNC0wMS0wNSIsImFjY2Vzc2VkRGF0ZSI6IjIwMjUtMTAiLCJjb250YWluZXJJbmZvIjp7ImF1dGhvcnMiOltdLCJ0aXRsZSI6IkZyb250aWVycyBpbiBFY29sb2d5IGFuZCBFdm9sdXRpb24iLCJ2b2x1bWUiOjExLCJpbXBhY3RGYWN0b3IiOiI0Ljc4In0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkZyb250aWVycyBNZWRpYSJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6NDAsImNpdGVkQnlDb3VudCI6NX19LCJxdW90ZSI6Im9nZW5pYyBudXRyaWVudCBpbnB1dCBzdGltdWxhdGVzIHN1cmZhY2UgZXV0cm9waGljYXRpb24gYW5kIGFsZ2FsIGJsb29tcyBhbmQgdGh1cyBleGFjZXJiYXRlcyBveHlnZW4gZGVwbGV0aW9uIGFuZCBoeXBveGlhIGluIGFxdWF0aWMgZWNvc3lzdGVtcy4gSG93ZXZlciwgZXhpc3Rpbmcgc3R1ZGllcyBoYXZlIHBhaWQgbGl0dGxlIGF0dGVudGlvbiBvbiB0aGUgdW5kZXJzdGFuZGluZyBvZiB0aGUgZmVlZGJhY2sgcmVsYXRpb25zaGlwcyBhbW9uZyBveHlnZW4gY29uc3VtcHRpb24sIGFsZ2FsIHN1Y2Nlc3Npb24gYW5kIGdyZWVuaG91c2UgZ2FzIGVmZmVjdCBhZnRlciByaXZlciBkYW1taW5nLCAuLi4gRE8gcGxheXMgYSBrZXkgcm9sZSBpbiB0aGUgYmlvZ2VvY2hlbWljYWwgY3ljbGUgb2Ygc3VyZmFjZSB3YXRlcnMgYW5kIGlzIGEgc2Vuc2l0aXZlIGluZGljYXRvciBvZiBwaHlzaWNhbCBhbmQgYmlvZ2VvY2hlbWljYWwgY2hhbmdlcyBpbiByZXNlcnZvaXIgYXF1YXRpYyBlY29zeXN0ZW1zICggV2V0emVsLCAyMDAxICkuIFdoZW4gd2F0ZXJzIGVudGVyIGFuIGFuYWVyb2JpYyBzdGF0ZSwgYXF1YXRpYyBvcmdhbmlzbXMgZGllIGluIGxhcmdlIG51bWJlcnMgYW5kIHRoZSByZWxlYXNlIG9mIHNlZGltZW50LXJlZHVjaW5nIHBvbGx1dGFudHMgKGUuZy4sIGFtbW9uaWEgbml0cm9nZW4sIC4uLiAyMDIyXSApLCBmdXJ0aGVyIGRldGVyaW9yYXRpbmcgd2F0ZXIgcXVhbGl0eS4gV2l0aCB0aGUgaW5jcmVhc2Ugb2YgcG9sbHV0aW9uIGxvYWRzIGluIHdhdGVyc2hlZHMgYW5kIHRoZSBpbnRlbnNpZmljYXRpb24gb2Ygd2F0ZXIgZXV0cm9waGljYXRpb24sIHdhdGVyIGh5cG94aWEgaGFzIGJlY29tZSBhIHNldmVyZSBnbG9iYWwgZWNvbG9naWNhbCBhbmQgZW52aXJvbm1lbnRhbCBwcm9ibGVtIGluIHJlc2Vydm9pcnMgKCBTdW4gZXTCoGFsLiwgMjAyMSA7IEphaXN3YWwgZXTCoGFsLiwgMjAyMyApLiBGb2N1c2luZyBvbiB0aGUgZHluYW1pYyBjaGFuZ2VzIGluIn1d.

Effective local waste management complements NBS by curbing point-source pollution from households and markets. Decentralized systems like community biogas digesters process organic waste into methane for cooking, while diverting sewage to constructed wetlands lined with gravel and reeds for filtration (Jilbert et al., 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlRvbSIsImZhbWlseSI6IkppbGJlcnQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUmFvdWzigJBNYXJpZSIsImZhbWlseSI6IkNvdXR1cmUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQnJpYW4iLCJmYW1pbHkiOiJIdXNlciJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLYWxldmkiLCJmYW1pbHkiOiJTYWxvbmVuIn19XSwiaWRzIjp7Imlzc24iOlsiMDAxOC04MTU4IiwiMTU3My01MTE3Il0sImlzYm4iOltdLCJkb2kiOiIxMC4xMDA3L3MxMDc1MC0wMjAtMDQ0NTcteCIsIm1hZyI6IjMxMDA0NTkwNTAiLCJvcGVuYWxleCI6IlczMTAwNDU5MDUwIn0sInRpdGxlIjoiUHJlZmFjZTogUmVzdG9yYXRpb24gb2YgZXV0cm9waGljIGxha2VzOiBjdXJyZW50IHByYWN0aWNlcyBhbmQgZnV0dXJlIGNoYWxsZW5nZXMiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAwNy9zMTA3NTAtMDIwLTA0NDU3LXgiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vbGluay5zcHJpbmdlci5jb20vY29udGVudC9wZGYvMTAuMTAwNy9zMTA3NTAtMDIwLTA0NDU3LXgucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMjAtMTEtMTYiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA3IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJIeWRyb2Jpb2xvZ2lhIiwiaXNzdWUiOjIxLCJ2b2x1bWUiOjg0NywiaW1wYWN0RmFjdG9yIjoiMy4wMCJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjQzNDMsImxhc3RQYWdlIjo0MzU3fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlNwcmluZ2VyIFNjaWVuY2UrQnVzaW5lc3MgTWVkaWEifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjk1LCJjaXRlZEJ5Q291bnQiOjU4fX0sInF1b3RlIjoiU2ltaWxhciBkZWdyYWRhdGlvbiBvZiBmcmVzaHdhdGVyIGVjb3N5c3RlbXMgaXMgb2JzZXJ2ZWQgd29ybGR3aWRlLCB3aXRoIHRoZSB0aW1lbGluZSBvZiBodW1hbiBwcmVzc3VyZSBkaWN0YXRlZCBieSB0cmVuZHMgaW4gdXJiYW5pemF0aW9uIGFuZCBpbnRlbnNpdmUgYWdyaWN1bHR1cmUgaW4gZGlmZmVyZW50IGdlb2dyYXBoaWMgcmVnaW9ucyAoZS5nLiwgVmVubmVtbyBldCBhbC4sIDIwMDkpLiBUaGUgcHJpbmNpcGFsIGRyaXZlciBvZiBlY29zeXN0ZW0gc3RyZXNzIGluIG1vc3QgYXF1YXRpYyBzeXN0ZW1zIGlzIGV1dHJvcGhpY2F0aW9uLCB0aGUgZXhjZXNzIGdyb3d0aCBvZiBhbGdhZSBkcml2ZW4gYnkgYW50aHJvcG9nZW5pYyBpbnB1dHMgb2YgdGhlIG1hY3JvbnV0cmllbnRzIG5pdHJvZ2VuIChOKSBhbmQgcGhvc3Bob3J1cyAoUCkgKFNtaXRoLCAyMDAzOyBCaXJrIGV0IGFsLiwgMjAyMCkuIE51dHJpZW50cyBhcmUgZW1pdHRlZCBmcm9tIGJvdGggcG9pbnQgYW5kIGRpZmZ1c2Ugc291cmNlcyBpbiB0aGUgaHVtYW4gZW52aXJvbm1lbnQgYW5kIGVudGVyIGFxdWF0aWMgc3lzdGVtcyB2aWEgc3VyZmFjZSBydW5vZmYsIGFzIHdlbGwgYXMgZ3JvdW5kLXdhdGVyIGFuZCBhdG1vc3BoZXJpYyBkZXBvc2l0aW9uIChQYWVybCwgMTk5NykuIEV1dHJvcGhpY2F0aW9uIG9mdGVuIGxlYWRzIHRvIHRoZSBkb21pbmFuY2Ugb2YgcHJpbWFyeSBwcm9kdWN0aW9uIGJ5IGN5YW5vYmFjdGVyaWEsIGluY2x1ZGluZyB0b3hpYyBnZW5lcmEgc3VjaCBhcyBEb2xpY2hvc3Blcm11bSwgQXBoYW5pem9tZW5vbiwgYW5kIE1pY3JvY3lzdGlzIChIdWlzbWFuIGV0IGFsLiwgMjAxOCksIGFzIHdlbGwgYXMgb3h5Z2VuIGRlcGxldGlvbiBhbmQgaGFiaXRhdCBkZWdyYWRhdGlvbiBpbiBsYWtlcyAoQXJlbmQgZXQgYWwuLCAyMDExKS4gQmVjYXVzZSBsYWtlcyBwcm92aWRlIGEgcmFuZ2Ugb2YgY3J1Y2lhbCBlY29zeXN0ZW0gc2VydmljZXMgKFdpbHNvbiAmIENhcnBlbnRlciwgMTk5OSksIHRoZXJlIGlzIGEgZ3Jvd2luZyBuZWVkIGZvciBtYW5hZ2VtZW50IGFwcHJvYWNoZXMgdGhhdCByZXN0b3JlIHdhdGVyIHF1YWxpdHkgaW4gZnJlc2h3YXRlciBzeXN0ZW1zLiBXaXRoaW4gdGhlIGZyYW1ld29yayBvZiBVbml0ZWQgTmF0aW9ucyBTdXN0YWluYWJsZSBEZXZlbG9wLW1lbnQgR29hbCA2IChDbGVhbiBXYXRlciBhbmQgU2FuaXRhdGlvbiksIHNldmVyYWwgZGlzdGluY3QgdGFyZ2V0cyByZWxhdGUgdG8gdGhlIGltcHJvdmVtZW50IG9mIHdhdGVyIHF1YWxpdHkgYW5kIHByb3RlY3Rpb24gb2YgYXF1YXRpYyBlY29zeXN0ZW1zIGJ5IDIwMzAgKFVuaXRlZCBOYXRpb25zLCAyMDE1KS4gIiwicGRmSGlnaGxpZ2h0cyI6eyJsb2NhdGlvbnMiOlt7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjoyOC4yOTg4NjQzNjQ2MjQwMjMsImxlZnQiOjguNTQ5MjA1NzgwMDI5Mjk3LCJ3aWR0aCI6MzkuMzcxNDk0MjkzMjEyODksImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6MjkuOTkxMTM0NjQzNTU0Njg4LCJsZWZ0Ijo4LjU0OTIwNTc4MDAyOTI5Nywid2lkdGgiOjM5LjM2NTExOTkzNDA4MjAzLCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjMxLjY4MzQwNjgyOTgzMzk4NCwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNjk2NjMyMzg1MjUzOSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozMy4zNjc5NzMzMjc2MzY3MiwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNzU2NzkwMTYxMTMyOCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNS4wNjAyNDkzMjg2MTMyOCwibGVmdCI6OC41NDkyMDU3ODAwMjkyOTcsIndpZHRoIjozOS4zNzA5NjQwNTAyOTI5NywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozNi43NTI1MTc3MDAxOTUzMSwibGVmdCI6OC41NDkyMDg2NDEwNTIyNDYsIndpZHRoIjozOS4zNzY5NjA3NTQzOTQ1MywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjozOC40MzcwODgwMTI2OTUzMSwibGVmdCI6OC41NDkyMDg2NDEwNTIyNDYsIndpZHRoIjozOS4zNjY1Njk1MTkwNDI5NywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0MC4xMjkzNTYzODQyNzczNDQsImxlZnQiOjguNTQ5MjA4NjQxMDUyMjQ2LCJ3aWR0aCI6MzkuMzY0Mzc5ODgyODEyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0MS44MjE2MjQ3NTU4NTkzNzUsImxlZnQiOjguNTQ5MjA4NjQxMDUyMjQ2LCJ3aWR0aCI6MzkuMzY1NjczMDY1MTg1NTUsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NDMuNTA2MTk1MDY4MzU5Mzc1LCJsZWZ0Ijo4LjU0OTM4OTgzOTE3MjM2Mywid2lkdGgiOjM5LjM3MDM5MTg0NTcwMzEyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0NS4xOTg0NjM0Mzk5NDE0MDYsImxlZnQiOjguNTQ5NTcyOTQ0NjQxMTEzLCJ3aWR0aCI6MzkuMzY1ODQ0NzI2NTYyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0Ni44OTA3MzU2MjYyMjA3LCJsZWZ0Ijo4LjU0OTU3Mjk0NDY0MTExMywid2lkdGgiOjM5LjM3Mjk1NTMyMjI2NTYyNSwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo0OC41NzUzMDIxMjQwMjM0NCwibGVmdCI6OC41NDk1NzI5NDQ2NDExMTMsIndpZHRoIjozOS4zNzI0MjEyNjQ2NDg0NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1MC4yNjc1NzgxMjUsImxlZnQiOjguNTQ5NTc2NzU5MzM4Mzc5LCJ3aWR0aCI6MzkuMzcxMTI0MjY3NTc4MTI1LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjUxLjk1OTg0MjY4MTg4NDc2NiwibGVmdCI6OC41NDk1NzY3NTkzMzgzNzksIndpZHRoIjozOS4zNjc4MzU5OTg1MzUxNTYsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTMuNTc0MTE5NTY3ODcxMDk0LCJsZWZ0Ijo4LjU0OTU3Njc1OTMzODM3OSwid2lkdGgiOjM5LjM3MDc3NzEzMDEyNjk1LCJoZWlnaHQiOjEuMzM2ODk1NTg1MDYwMTE5Nn19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjU1LjI2NjM5MTc1NDE1MDM5LCJsZWZ0Ijo4LjU0OTc2MDgxODQ4MTQ0NSwid2lkdGgiOjM5LjM2NjU4MDk2MzEzNDc2NiwiaGVpZ2h0IjoxLjMzNjg5NTU4NTA2MDExOTZ9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo1Ny4wMjg5NTM1NTIyNDYwOTQsImxlZnQiOjguNTQ5OTQ0ODc3NjI0NTEyLCJ3aWR0aCI6MzkuMzYyMDI2MjE0NTk5NjEsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NTguNzEzNTIzODY0NzQ2MDk0LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjkuMjk5OTgyMDcwOTIyODUyLCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjYwLjQwNTc5MjIzNjMyODEyNSwibGVmdCI6MTAuNjIyNDU1NTk2OTIzODI4LCJ3aWR0aCI6MzcuMjg3ODY4NDk5NzU1ODYsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX0seyJwYWdlTnVtYmVyIjoxLCJiYm94Ijp7InRvcCI6NjIuMDk4MDYwNjA3OTEwMTU2LCJsZWZ0Ijo4LjU0OTk0MzkyMzk1MDE5NSwid2lkdGgiOjM5LjM3MzMyNTM0NzkwMDM5LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjYzLjc4MjYzMDkyMDQxMDE1NiwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzMxMjY5ODM2NDI1OCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2NS40NzQ5MDY5MjEzODY3MiwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNjUxMTk5MzQwODIwMywiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2Ny4xNjcxNzUyOTI5Njg3NSwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNjk0ODAxMzMwNTY2NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo2OC43ODE0NDgzNjQyNTc4MSwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzczMzg0MDk0MjM4MywiaGVpZ2h0IjoxLjMzNjg5MTUzMTk0NDI3NX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjcwLjU0NDAxMzk3NzA1MDc4LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjM5LjM2OTMwODQ3MTY3OTY5LCJoZWlnaHQiOjEuMjY2NjAyMzk2OTY1MDI2OX19LHsicGFnZU51bWJlciI6MSwiYmJveCI6eyJ0b3AiOjcyLjIzNjI4OTk3ODAyNzM0LCJsZWZ0Ijo4LjU0OTk0NDg3NzYyNDUxMiwid2lkdGgiOjM5LjM3MjAzMjE2NTUyNzM0NCwiaGVpZ2h0IjoxLjI2NjYwMjM5Njk2NTAyNjl9fSx7InBhZ2VOdW1iZXIiOjEsImJib3giOnsidG9wIjo3My45MjA4NDUwMzE3MzgyOCwibGVmdCI6OC41NDk5NDQ4Nzc2MjQ1MTIsIndpZHRoIjozOS4zNzEzMDczNzMwNDY4NzUsImhlaWdodCI6MS4yNjY2MDIzOTY5NjUwMjY5fX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiNDM0NCJ9fV0%3D. In Ralegan Siddhi, contour bunding and tree pits halved erosion into lakes, a model replicable with low-cost kits validating bloom indicators like fish kills (Foulon et al., 2019)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IsOJdGllbm5lIiwiZmFtaWx5IjoiRm91bG9uIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFsYWluIE4uIiwiZmFtaWx5IjoiUm91c3NlYXUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiR2xlbm4iLCJmYW1pbHkiOiJCZW5veSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJSZWJlY2NhIEwuIiwiZmFtaWx5IjoiTm9ydGgifX1dLCJpZHMiOnsiaXNzbiI6WyIxMjAxLTMwODAiLCIyNDA4LTk0NDMiXSwiaXNibiI6W10sImRvaSI6IjEwLjIxNjYvd3Fyai4yMDE5LjAxMyIsIm1hZyI6IjI5NzM4NjAyODEiLCJvcGVuYWxleCI6IlcyOTczODYwMjgxIn0sInRpdGxlIjoiQSBnbG9iYWwgc2NhbiBvZiBob3cgdGhlIGlzc3VlIG9mIG51dHJpZW50IGxvYWRpbmcgYW5kIGhhcm1mdWwgYWxnYWwgYmxvb21zIGlzIGJlaW5nIGFkZHJlc3NlZCBieSBnb3Zlcm5tZW50cywgbm9uLWdvdmVybm1lbnRhbCBvcmdhbml6YXRpb25zLCBhbmQgdm9sdW50ZWVycyIsImFic3RyYWN0IjoiQWJzdHJhY3QgSGFybWZ1bCBhbGdhbCBibG9vbXMgKEhBQnMpIGluIGFxdWF0aWMgZWNvc3lzdGVtcyBhcmUgb2YgY29uY2VybiB3b3JsZHdpZGUuIFRoaXMgcmV2aWV3IGRlYWxzIHdpdGggaG93IGp1cmlzZGljdGlvbnMgYXJvdW5kIHRoZSB3b3JsZCBhcmUgYWRkcmVzc2luZyB0aGlzIHdhdGVyIHF1YWxpdHkgaXNzdWUgdG8gaW5mb3JtIHJlY29tbWVuZGF0aW9ucyByZWdhcmRpbmcgbnV0cmllbnQgbG9hZGluZyBhbmQgSEFCcyBpbiBNaXNzaXNxdW9pIEJheS1MYWtlIENoYW1wbGFpbiBhbmQgTGFrZSBNZW1waHJlbWFnb2c7IHRyYW5zYm91bmRhcnkgbGFrZXMgbG9jYXRlZCBpbiB0aGUgVVNBIGFuZCBDYW5hZGEgdGhhdCBzdWZmZXIgZnJvbSBzeW1wdG9tcyBvZiBldXRyb3BoaWNhdGlvbi4gQSBnbG9iYWwgc2NhbiBvZiB0aGUgbGl0ZXJhdHVyZSByZXN1bHRlZCBpbiB0aGUgY29uc2lkZXJhdGlvbiBvZiAxMiBjYXNlIHN0dWRpZXMgb2YgbGFyZ2Ugd2F0ZXIgYm9kaWVzIHdpdGhpbiBsYXJnZSB3YXRlcnNoZWRzLCBleGNsdWRpbmcgaW4tbGFrZSBnZW9lbmdpbmVlcmluZyBhcHByb2FjaGVzLiBBbHRob3VnaCBhbGwgb2YgdGhlIHN5c3RlbXMgZXhwZXJpZW5jZSBleGNlc3NpdmUgbnV0cmllbnQgbG9hZGluZywgdGhleSB2YXJ5IGluIHR3byBrZXkgd2F5czogc291cmNlcyBvZiBudXRyaWVudHMgYW5kIG1hbmlmZXN0YXRpb25zIG9mIGV1dHJvcGhpY2F0aW9uIHJhbmdpbmcgZnJvbSBIQUJzLCB0byBsaW1pdGVkIHJlY3JlYXRpb25hbCB1c2VzLCB0byB0aGUgYWRkaXRpb25hbCBjb21wbGV4aXR5IG9mIGludGVybmFsIGxvYWRpbmdzIGFuZCBmaXNoIGtpbGxzLCB1cCB0byBkcmlua2luZyB3YXRlciBzaHV0ZG93bnMuIFRoZSBjYXNlIHN0dWRpZXMgd2VyZSBhbmFseXplZCB3aXRoIHJlc3BlY3QgdG8gZm91ciBjYXRlZ29yaWVzIG9mIGFwcHJvYWNoZXMsIG5hbWVseTogKGkpIHJlZ3VsYXRvcnk7IChpaSkgaW5jZW50aXZlLWJhc2VkOyAoaWlpKSByaXNrIG1pdGlnYXRpb247IGFuZCAoaXYpIG91dHJlYWNoLCBlbmdhZ2VtZW50LCBhbmQgZWR1Y2F0aW9uYWwuIFdlIGZvdW5kIHRoYXQgdGhlIG1hbmFnZW1lbnQgZnJhbWV3b3JrcyBhcmUgYmFzZWQgb24gaW50ZWdyYXRlZCB3YXRlcnNoZWQgbWFuYWdlbWVudCBwbGFubmluZyBhbmQgbmF0aW9uYWwgc3RhbmRhcmRzLiBOYXRpb25hbCB3YXRlciBxdWFsaXR5IHN0YW5kYXJkcywgaG93ZXZlciwgYXJlIG5vdCBzdHJpbmdlbnQgZW5vdWdoIHRvIHByZXZlbnQgSEFCcy4gT3ZlcmFsbCwgaWRlbnRpZmllZCBjYXNlIHN0dWRpZXMgZGlkIG5vdCBzdWNjZXNzZnVsbHkgcmVtZWRpYXRlIEhBQnMsIHRoZXkgc2ltcGx5IG1hbmFnZWQgdGhlbS4iLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMjE2Ni93cXJqLjIwMTkuMDEzIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2l3YXBvbmxpbmUuY29tL3dxcmovYXJ0aWNsZS1wZGYvNTUvMS8xLzcwOTI2MS93cXJqYzA1NTAwMDEucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMTktMDktMTciLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTA5IiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJXYXRlciBRdWFsaXR5IFJlc2VhcmNoIEpvdXJuYWwiLCJpc3N1ZSI6MSwidm9sdW1lIjo1NSwiaW1wYWN0RmFjdG9yIjoiNy44MyJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjEsImxhc3RQYWdlIjoyM319LCJwdWJsaXNoZXJJbmZvIjp7Im5hbWUiOiJVV0EgUHVibGlzaGluZyJ9LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6ODQsImNpdGVkQnlDb3VudCI6MjB9fSwicXVvdGUiOiJXZSBwcm9wb3NlIHNvbHV0aW9ucyBhbmQgcGF0aHMgZm9yd2FyZCB0byBhIHdpY2tlZCBwcm9ibGVtIHdoaWNoIHNob3VsZCBwcm92ZSBpbnNpZ2h0ZnVsIGZvciBwb2xpY3ktbWFrZXJzIGFuZCBtYW4tIGFnZXJzIGFsaWtlIHRocm91Z2ggdGhlIGZvbGxvd2luZyBmaW5kaW5ncy4gTmF0aW9uYWwgd2F0ZXIgcXVhbGl0eSBzdGFuZGFyZHMgaGF2ZSBub3QgcHJvdmVkIHN0cmluZ2VudCBlbm91Z2ggdG8gcHJlLSB2ZW50IEhBQnMuIElkZW50aWZpZWQgZnJhbWV3b3JrcyBhcmUgYWxsIGJhc2VkIG9uIGludGVncmF0ZWQgd2F0ZXJzaGVkIG1hbmFnZW1lbnQgYW5kIG5hdGlvbmFsIHN0YW5kYXJkcy4gTm8gYWN0dWFsIHN1Y2Nlc3Mgc3RvcmllcyByZWxhdGVkIHRvIHJlbWVkaWF0aW9uIG9mIEhBQnMgd2VyZSBpZGVudGlmaWVkOyB0aGV5IHdlcmUgc2ltcGx5IG1hbmFnZW1lbnQgc3Rvcmllcy4gVGhlIGVmZmljYWN5IG9mIE5QUyBCTVBzIGlzIHF1ZXN0aW9uZWQgdGhyb3VnaG91dCB0aGUgd29ybGQgZHVlIHRvIGluc3VmZmljaWVudCBldmlkZW5jZSB0byBkZXRlcm1pbmUgd2hldGhlciB0aGV5IGNhbiBtaXRpZ2F0ZSBIQUJzLiBNYXJrZXQtYmFzZWQgc3RyYXRlZ2llcyB0byBvZmZzZXQgcG9sbHV0aW9uIG9yIHByb21vdGUgdGFyZ2V0ZWQgQk1QIGFkb3B0aW9uIGFyZSBiZWNvbWluZyBwb3B1bGFyIGFuZCByZXN1bHRpbmcgaW4gY29zdC1lZmZlY3RpdmUgYW5kIGVmZmljaWVudCBhcHByb2FjaGVzLiBDb3N04oCTYmVuZWZpdCBhbmFseXNlcyBtYXkgaW5jcmVhc2UgdGhlIHVzZSBvZiB0ZWNobmljYWwgc29sLSB1dGlvbnMgb3ZlciBjb21wcmVoZW5zaXZlIGFwcHJvYWNoZXMuIEF0IHRoaXMgcG9pbnQsIHRoZXJlIGlzIG5vdCByZWFsbHkgYSB1bmlxdWUgY2FzZSBzdHVkeSB0aGF0IGNvdWxkIGJlIGFwcGxpZWQgdG8gTGFrZSBDaGFtcGxhaW4gYW5kIExha2UgTWVtcGhyZW1hZ29nLiBTcGVjaWZpYyBlbGVtZW50cyBvZiB0aGUgYXBwcm9hY2hlcyBpZGVudGlmaWVkIGluIHRoaXMgcmV2aWV3IGFuZCB1bmktIGZ5aW5nIHRoZW1lcywgaG93ZXZlciwgaG9sZCBwcm9taXNlcy4gVGhpcyBpcyB3aHkgd2UgYWR2b2NhdGUgZm9yIGNvbmR1Y3RpbmcgYSBzeXN0ZW1hdGljIGxpdGVyYXR1cmUgcmV2aWV3IHdpdGggcmVnYXJkIHRvIHRob3NlIHRoZW1lcyBpbiBvcmRlciB0byBkZXZlbG9wIHJlY29tbWVuLSBkYXRpb25zIHRhaWxvcmVkIHRvIHRoZSBMYWtlIE1lbXBocmVtYWdvZyBhbmQgTWlzc2lzcXVvaSBCYXktTGFrZSBDaGFtcGxhaW4gYXJlYXMuIEF0IGxhc3QsIHdlIHdvdWxkIGxpa2UgdG8gYWRkcmVzcyB0aGUgZWxlcGhhbnQtc2l6ZWQgcXVlcy0gdGlvbiBpbiB0aGUgcm9vbS4gV2lsbCBjbGltYXRlIGNoYW5nZSBpbmNyZWFzZSBIQUJzIGZyZXF1ZW5jeT8gSW50ZXJhY3RpdmUgZWZmZWN0cyBvZiBldXRyb3BoaWNhdGlvbiBhbmQgY2xpbWF0ZSBjaGFuZ2Ugb24gaGFybWZ1bCBjeWFub2JhY3RlcmlhbCBibG9vbXMgYXJlIGNvbXBsZXguICIsInBkZkhpZ2hsaWdodHMiOnsibG9jYXRpb25zIjpbeyJwYWdlTnVtYmVyIjoxNSwiYmJveCI6eyJ0b3AiOjg1LjYwNTUxNDUyNjM2NzE5LCJsZWZ0Ijo0OS43OTQ4NzIyODM5MzU1NSwid2lkdGgiOjM5LjcxODY2MjI2MTk2Mjg5LCJoZWlnaHQiOjEuMzI4NjcxMzM2MTc0MDExMn19LHsicGFnZU51bWJlciI6MTUsImJib3giOnsidG9wIjo4Ny4zOTUzNjI4NTQwMDM5LCJsZWZ0Ijo0OS43OTQ4NzIyODM5MzU1NSwid2lkdGgiOjM5LjcwMjcwOTE5Nzk5ODA1LCJoZWlnaHQiOjEuMzI4NjcxMzM2MTc0MDExMn19LHsicGFnZU51bWJlciI6MTUsImJib3giOnsidG9wIjo4OS4xODUyMTExODE2NDA2MiwibGVmdCI6NDkuNzk0ODcyMjgzOTM1NTUsIndpZHRoIjozOS43MDIwOTUwMzE3MzgyOCwiaGVpZ2h0IjoxLjMyODY3MTMzNjE3NDAxMTJ9fSx7InBhZ2VOdW1iZXIiOjE1LCJiYm94Ijp7InRvcCI6OTAuOTc1MDU5NTA5Mjc3MzQsImxlZnQiOjQ5Ljc5NDg3MjI4MzkzNTU1LCJ3aWR0aCI6MzkuNzA0NjIwMzYxMzI4MTI1LCJoZWlnaHQiOjEuMzI4NjcxMzM2MTc0MDExMn19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjoxNC4wMjU5OTA0ODYxNDUwMiwibGVmdCI6MTAuNTAyMzIzMTUwNjM0NzY2LCJ3aWR0aCI6MzkuNzA1OTU1NTA1MzcxMDk0LCJoZWlnaHQiOjEuMzI4NjY5NDI4ODI1Mzc4NH19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjoxNS44MTU4NDE2NzQ4MDQ2ODgsImxlZnQiOjEwLjUwMjMyMzE1MDYzNDc2Niwid2lkdGgiOjM5LjcwMjI0NzYxOTYyODkwNiwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MTcuNjA1NjkxOTA5NzkwMDQsImxlZnQiOjEwLjUwMjE3NjI4NDc5MDAzOSwid2lkdGgiOjM5LjcwMDkyMzkxOTY3NzczNCwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MTkuMzk1NTQ0MDUyMTI0MDIzLCJsZWZ0IjoxMC41MDIxNzYyODQ3OTAwMzksIndpZHRoIjozOS43MDg5MDA0NTE2NjAxNTYsImhlaWdodCI6MS4zMjg2Njk0Mjg4MjUzNzg0fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjIxLjE4NTM5NDI4NzEwOTM3NSwibGVmdCI6MTAuNTAyMTc2Mjg0NzkwMDM5LCJ3aWR0aCI6MzkuNzEwMDgzMDA3ODEyNSwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MjIuOTc1MjQ0NTIyMDk0NzI3LCJsZWZ0IjoxMC41MDIxNzYyODQ3OTAwMzksIndpZHRoIjozOS43MDQwMjkwODMyNTE5NSwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MjQuNzY1MDk2NjY0NDI4NzEsImxlZnQiOjEwLjUwMjE3NjI4NDc5MDAzOSwid2lkdGgiOjM5LjcwMzg4MDMxMDA1ODU5NCwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MjYuNTU0OTQ4ODA2NzYyNjk1LCJsZWZ0IjoxMC41MDIwMjc1MTE1OTY2OCwid2lkdGgiOjM5LjcwMzg4MDMxMDA1ODU5NCwiaGVpZ2h0IjoxLjMyODY2OTQyODgyNTM3ODR9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6MjguMzQ0Nzk5MDQxNzQ4MDQ3LCJsZWZ0IjoxMC41MDIwMjc1MTE1OTY2OCwid2lkdGgiOjM5LjcwMDAzODkwOTkxMjExLCJoZWlnaHQiOjEuMzI4NjY5NDI4ODI1Mzc4NH19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjozMC4xMzQ2NDczNjkzODQ3NjYsImxlZnQiOjEwLjUwMjAyNzUxMTU5NjY4LCJ3aWR0aCI6MzkuNzA0NjE2NTQ2NjMwODYsImhlaWdodCI6MS4zMjg2Njk0Mjg4MjUzNzg0fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjMxLjkyNDQ5NzYwNDM3MDExNywibGVmdCI6MTAuNTAxODc3Nzg0NzI5MDA0LCJ3aWR0aCI6MzkuNzA1Nzg3NjU4NjkxNDA2LCJoZWlnaHQiOjEuMzI4NjY5NDI4ODI1Mzc4NH19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjozMy43MTQzNTE2NTQwNTI3MzQsImxlZnQiOjEwLjUwMTcyOTAxMTUzNTY0NSwid2lkdGgiOjM5LjcxOTI0NTkxMDY0NDUzLCJoZWlnaHQiOjEuMzI4NjY3NDAyMjY3NDU2fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjM1LjUwNDIwMzc5NjM4NjcyLCJsZWZ0IjoxMC41MDE3MjkwMTE1MzU2NDUsIndpZHRoIjozOS43MDI3MDkxOTc5OTgwNSwiaGVpZ2h0IjoxLjMyODY2NzQwMjI2NzQ1Nn19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjozNy4yOTQwNTIxMjQwMjM0NCwibGVmdCI6MTAuNTAxNzI5MDExNTM1NjQ1LCJ3aWR0aCI6MzkuNzA1Nzk5MTAyNzgzMiwiaGVpZ2h0IjoxLjMyODY2NzQwMjI2NzQ1Nn19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjozOS4wODM5MDQyNjYzNTc0MiwibGVmdCI6MTAuNTAxNzI1MTk2ODM4Mzc5LCJ3aWR0aCI6MzkuNzA2NTM1MzM5MzU1NDcsImhlaWdodCI6MS4zMjg2Njc0MDIyNjc0NTZ9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6NDAuODczNzU2NDA4NjkxNDA2LCJsZWZ0IjoxMC41MDE3MjcxMDQxODcwMTIsIndpZHRoIjozOS43MjExNTMyNTkyNzczNDQsImhlaWdodCI6MS4zMjg2Njc0MDIyNjc0NTZ9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6NDIuNjYzNjA4NTUxMDI1MzksImxlZnQiOjEwLjUwMTcyNzEwNDE4NzAxMiwid2lkdGgiOjM5LjcwMzEzNjQ0NDA5MTgsImhlaWdodCI6MS4zMjg2Njc0MDIyNjc0NTZ9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6NDQuNDQ2MjA4OTUzODU3NDIsImxlZnQiOjEwLjUwMTcyNzEwNDE4NzAxMiwid2lkdGgiOjM5LjcwMDc3NTE0NjQ4NDM3NSwiaGVpZ2h0IjoxLjMyODY2NzQwMjI2NzQ1Nn19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjo0Ni4yMzYwNjEwOTYxOTE0MDYsImxlZnQiOjEwLjUwMTcyNzEwNDE4NzAxMiwid2lkdGgiOjM5LjcwNzcyNTUyNDkwMjM0NCwiaGVpZ2h0IjoxLjMyODY2NzQwMjI2NzQ1Nn19LHsicGFnZU51bWJlciI6MTYsImJib3giOnsidG9wIjo0OC4wMjU5MTMyMzg1MjUzOSwibGVmdCI6MTAuNTAxNzI3MTA0MTg3MDEyLCJ3aWR0aCI6MjUuOTQxNjY3NTU2NzYyNjk1LCJoZWlnaHQiOjEuMzI4NjY3NDAyMjY3NDU2fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjQ5LjgxNTc2MTU2NjE2MjExLCJsZWZ0IjoxMy4xMjUwMTYyMTI0NjMzNzksIndpZHRoIjozNy4wODExODA1NzI1MDk3NjYsImhlaWdodCI6MS4zMjg2Njc0MDIyNjc0NTZ9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6NTEuNjA1NjEzNzA4NDk2MDk0LCJsZWZ0IjoxMC41MDE3MjcxMDQxODcwMTIsIndpZHRoIjozOS43MDkyMDU2Mjc0NDE0MDYsImhlaWdodCI6MS4zMjg2Njc0MDIyNjc0NTZ9fSx7InBhZ2VOdW1iZXIiOjE2LCJiYm94Ijp7InRvcCI6NTMuMzk1NDYyMDM2MTMyODEsImxlZnQiOjEwLjUwMTcyNzEwNDE4NzAxMiwid2lkdGgiOjM5LjcxOTIyMzAyMjQ2MDk0LCJoZWlnaHQiOjEuMzI4NjY3NDAyMjY3NDU2fX0seyJwYWdlTnVtYmVyIjoxNiwiYmJveCI6eyJ0b3AiOjU1LjE4NTMxMDM2Mzc2OTUzLCJsZWZ0IjoxMC41MDE3MjcxMDQxODcwMTIsIndpZHRoIjozOS43MTA2NzgxMDA1ODU5NCwiaGVpZ2h0IjoxLjMyODY2NzQwMjI2NzQ1Nn19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6IjE2In19XQ%3D%3D. Economic incentives, such as fisheries offsets, reward verified reductions, recouping tourism losses estimated at INR 500-1000 crore annually in Bengaluru (Berdalet et al., 2017; Devlin et al., 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjEsInN1YnR5cGUiOnt9LCJjb250cmlidXRvcnMiOlt7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBZGFtIFQuIiwiZmFtaWx5IjoiRGV2bGluIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkppYXlpIiwiZmFtaWx5IjoiUGFuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik0uIE1hbmp1ciIsImZhbWlseSI6IlNoYWgifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQWxpeXUiLCJmYW1pbHkiOiJOdWh1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Iktlbm5ldGggVy4iLCJmYW1pbHkiOiJDdW1taW5zIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkNsYXVkaW8gVmFsZG92aW5vcyIsImZhbWlseSI6IlphcmdlcyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQYWJsbyIsImZhbWlseSI6IkZpZXJybyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJWaXZpYW5hIiwiZmFtaWx5IjoiT2xtb3MifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQWJkdWwtUmFobWFuIiwiZmFtaWx5IjoiRGlyaXN1In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkpvaG4gT3ZpZSIsImZhbWlseSI6Ik9sb211a29ybyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJOYXRpdmlkYWRlIiwiZmFtaWx5IjoiVmllaXJhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFuYSIsImZhbWlseSI6IlNvYXJlcyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJDYXJsb3MiLCJmYW1pbHkiOiJQaW5oZWlybyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNYXR0aGV3IEYuIiwiZmFtaWx5IjoiQmluZ2hhbSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJKYXNvbiIsImZhbWlseSI6Iktpbm5lbGwifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiV3UiLCJmYW1pbHkiOiJCbyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJKaW5tdSIsImZhbWlseSI6IlpoYW5nIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IllpbmRpIiwiZmFtaWx5IjoiWmhhbyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJIdW1iZXJ0byBBbHZlcyIsImZhbWlseSI6IkJhcmJvc2EifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiRnJhbmtsaW4iLCJmYW1pbHkiOiJQYXJlZGVz4oCQVHJlam8ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiVCBMYWtzaG1pIiwiZmFtaWx5IjoiS3VtYXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiS2lyYW4iLCJmYW1pbHkiOiJUaGFrdXIifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQ2F0YXJpbmEiLCJmYW1pbHkiOiJCdXJpdGkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2NvdHQgQS4iLCJmYW1pbHkiOiJXZWxscyJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJPc2NhciBPbW9uZGkiLCJmYW1pbHkiOiJEb25kZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJDYXJlbiBOYXZhbGlhIiwiZmFtaWx5IjoiQXRhbGl0c2EifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiRC4iLCJmYW1pbHkiOiJTYW5pIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNhbXBzb24iLCJmYW1pbHkiOiJBaG1lZCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJBbWFuaW5nIiwiZmFtaWx5IjoiS3dha3UifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiR2V2b3JnIiwiZmFtaWx5IjoiQWRqZWkifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2F0YXJ1cGEiLCJmYW1pbHkiOiJTaW1vbnlhbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJTbmlnZGhhIiwiZmFtaWx5IjoiR2hvc2gifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUHJhc2FubmEiLCJmYW1pbHkiOiJDaGF0dGVyamVlIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNoaXZhIiwiZmFtaWx5IjoiR2hvcmEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQmVsZGEiLCJmYW1pbHkiOiJQcmFzYWQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiS2V0aGxhdGxvZ2lsZSIsImZhbWlseSI6Ik1vc2VwZWxlIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ill1c3VmIiwiZmFtaWx5IjoiU3VsZSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJVaXLDoSIsImZhbWlseSI6Ik9saXZlaXJhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik1hcmlhIEpvw6NvIiwiZmFtaWx5IjoiVmllaXJhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkZsZXVyIiwiZmFtaWx5IjoiRGUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTWluIiwiZmFtaWx5IjoiTWF4In19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ikdldm9yZyIsImZhbWlseSI6IlNpbW9ueWFuIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlNhdGFydXBhIiwiZmFtaWx5IjoiR2hvc2gifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU25pZ2RoYSIsImZhbWlseSI6IkNoYXR0ZXJqZWUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiUHJhc2FubmEiLCJmYW1pbHkiOiJQcmFzYWQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiU2F0YXJ1cGEiLCJmYW1pbHkiOiJHaG9zaCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJHaG9yYSIsImZhbWlseSI6IlByYXNhZCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQcmFzYW5uYSIsImZhbWlseSI6IlBhbCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLd2FrdSIsImZhbWlseSI6IkFnb2R6byJ9fV0sImlkcyI6eyJpc3NuIjpbXSwiaXNibiI6W10sImRvaSI6IjEwLjU3NzIvaW50ZWNob3Blbi44NzQ2MyIsIm1hZyI6IjMxMjg1ODc0NjkiLCJvcGVuYWxleCI6IlczMTI4NTg3NDY5In0sInRpdGxlIjoiSW5sYW5kIFdhdGVycyAtIER5bmFtaWNzIGFuZCBFY29sb2d5IiwiYWJzdHJhY3QiOiJJbmxhbmQgd2F0ZXJzLCBsYWtlcywgcml2ZXJzLCBhbmQgdGhlaXIgY29ubmVjdGVkIHdldGxhbmRzIGFyZSB0aGUgbW9zdCBpbXBvcnRhbnQgYW5kIHRoZSBtb3N0IHZ1bG5lcmFibGUgc291cmNlcyBvZiBmcmVzaHdhdGVyIG9uIHRoZSBwbGFuZXQuIFRoZSBlY29sb2d5IG9mIHRoZXNlIHN5c3RlbXMgaW5jbHVkZXMgYmlvbG9neSBhcyB3ZWxsIGFzIGh1bWFuIHBvcHVsYXRpb25zIGFuZCBjaXZpbGl6YXRpb24uIElubGFuZCB3YXRlcnMgYW5kIHdldGxhbmRzIGFyZSBoaWdobHkgc3VzY2VwdGlibGUgdG8gY2hlbWljYWwgYW5kIGJpb2xvZ2ljYWwgcG9sbHV0YW50cyBmcm9tIG5hdHVyYWwgb3IgaHVtYW4gc291cmNlcywgY2hhbmdlcyBpbiB3YXRlcnNoZWQgZHluYW1pY3MgZHVlIHRvIHRoZSBlc3RhYmxpc2htZW50IG9mIGRhbXMgYW5kIHJlc2Vydm9pcnMsIGFuZCBsYW5kIHVzZSBjaGFuZ2VzIGZyb20gYWdyaWN1bHR1cmUgYW5kIGluZHVzdHJ5LiBUaGlzIGJvb2sgcHJvdmlkZXMgYSBjb21wcmVoZW5zaXZlIHJldmlldyBvZiBpc3N1ZXMgaW52b2x2aW5nIGlubGFuZCB3YXRlcnMgYW5kIGRpc2N1c3NlcyBtYW55IHdvcmxkd2lkZSBpbmxhbmQgd2F0ZXIgc3lzdGVtcy4gVGhlIG1haW4gdG9waWNzIG9mIHRoaXMgdGV4dCBhcmUgd2F0ZXIgcXVhbGl0eSBpbnZlc3RpZ2F0aW9uLCBhbmFseXNlcyBvZiB0aGUgZWNvbG9neSBvZiBpbmxhbmQgd2F0ZXIgc3lzdGVtcywgcmVtb3RlIHNlbnNpbmcgb2JzZXJ2YXRpb24gYW5kIG51bWVyaWNhbCBtb2RlbGluZyBtZXRob2RzLCBhbmQgYmlvZGl2ZXJzaXR5IGludmVzdGlnYXRpb25zLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC41NzcyL2ludGVjaG9wZW4uODc0NjMiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vbXRzLmludGVjaG9wZW4uY29tL3N0b3JhZ2UvYm9va3MvOTY2MC9hdXRob3JzX2Jvb2svYXV0aG9yc19ib29rLnBkZiIsImlzc3VlZERhdGUiOiIyMDIwLTA0LTAxIiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMCIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiSW50ZWNoT3BlbiBlQm9va3MiLCJpbXBhY3RGYWN0b3IiOiIxLjAxIn0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkludGVjaE9wZW4ifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjMzOCwiY2l0ZWRCeUNvdW50IjoyN319LCJxdW90ZSI6IkFkZGl0aW9uYWxseSwgc2luY2UgYWZmZWN0ZWQgYnVzaW5lc3Nlc%2BKAmSB2YWx1ZXMgIGFyZSBtb3N0IGxpa2VseSB0aWVkIHRvIHRoZWlyIGFzc2V0cyBhbmQgdGhlIHJlYWwgZXN0YXRlIHRoZXkgb2NjdXB5IChmb3IgZXhhbXBsZSwgIGEgbWFyaW5hIGlzIG5vdCBlYXNpbHkgY29udmVydGVkIHRvIHNvbWUgb3RoZXIgdXNlKSwgb24tZ29pbmcgYmFsYW5jZSBzaGVldCBlZmZlY3RzICB3b3VsZCB1bHRpbWF0ZWx5IGxlYWQgdG8gcmVkdWN0aW9ucyBpbiBjb21tZXJjaWFsIHJlYWwgZXN0YXRlIHZhbHVlcy4gVGhlcmUgYXJlIG1hbnkgY2hhbGxlbmdlcyB0byB1bmRlcnN0YW5kaW5nIHRoZSBpbXBsaWNhdGlvbnMgb2YgY2hhbmdlcyBpbiAgdG91cmlzbSBmcm9tIEhBQnMuIFRoZSBjbGVhcmVzdCBjaGFsbGVuZ2Ugb2JzdHJ1Y3RpbmcgYSBwcmVjaXNlIGFuYWx5c2lzIG9mIHRoZXNlICBpbXBhY3RzIGlzIGEgbGFjayBvZiBkYXRhIGVpdGhlciBvbiB0aGUgYW1vdW50IG9mIHRvdXJpc20gYXQgcmlzayBvciB0aGUgc3BlY2lmaWMgIGltcGFjdCBvZiBIQUJzIG9uIHRvdXJpc20uIEZvciBleGFtcGxlLCB3aGlsZSBjb3VudHktbGV2ZWwgZGF0YSBleGlzdHMgZm9yIHRvdGFsICBleHBlbmRpdHVyZXMgb24gdG91cmlzbSwgdGhpcyBpbmNsdWRlcyB0b3VyaXNtIHdoaWNoIHdvdWxkIG5vdCBiZSBpbnRlcnJ1cHRlZCBieSAgSEFCcyBvciBvdGhlciBkaXNjb3VyYWdpbmcgZmFjdG9ycy4gQW4gYWRkaXRpb25hbCBjaGFsbGVuZ2UgcmVsYXRlcyB0byB0aGUgZGlzdGluY3Rpb24gYmV0d2VlbiBlY29ub21pYyBiZW5lZml0cyAgKHdpbGxpbmduZXNzIHRvIHBheSkgYW5kIGVjb25vbWljIGltcGFjdHMgKGV4cGVuZGl0dXJlcyksIGFuZCB0aGUgbWVhc3VyZS1tZW50IG9mIHRoZSBlY29ub21pYyBiZW5lZml0cyB0aGF0IGFyaXNlIGZyb20gZWNvbm9taWMgaW1wYWN0cyAocHJvZml0cykuIEZvciAgZXhhbXBsZSwgY29uc2lkZXIgYSByZXN0YXVyYW50IG93bmVyIHdobyBsb3NlcyAkMTAsMDAwIGluIHJldmVudWUgYmVjYXVzZSBvZiBhICBIQUIuIFRoZSBvd25lcuKAmXMgd2lsbGluZ25lc3MgdG8gcGF5IHRvIHJlY292ZXIgdGhhdCByZXZlbnVlOyBpcyAocm91Z2hseSBzcGVha2luZykgIHRoZSBsb3N0IHByb2ZpdCBvbiB0aGF0IHJldmVudWUuIFRoaXMgaXMgbW9yZSBkaWZmaWN1bHQgdG8gaWRlbnRpZnkgdGhhbiBsb3N0IHJldmVudWUuICAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6ODksImJib3giOnsidG9wIjo1OS40NDI4NjcyNzkwNTI3MzQsImxlZnQiOjE0LjcyMjg1MzY2MDU4MzQ5Niwid2lkdGgiOjY4Ljc0ODAwMTA5ODYzMjgxLCJoZWlnaHQiOjEuOTA1MDAwODA1ODU0Nzk3NH19LHsicGFnZU51bWJlciI6ODksImJib3giOnsidG9wIjo2MS4wNzEwNzU0Mzk0NTMxMjUsImxlZnQiOjE0LjcyMjg1MzY2MDU4MzQ5Niwid2lkdGgiOjY2LjM5NjM0NzA0NTg5ODQ0LCJoZWlnaHQiOjEuOTA1MDAwODA1ODU0Nzk3NH19LHsicGFnZU51bWJlciI6ODksImJib3giOnsidG9wIjo2Mi42OTkyNzU5NzA0NTg5ODQsImxlZnQiOjE0LjcyMjg1MzY2MDU4MzQ5Niwid2lkdGgiOjY5LjI2OTMwOTk5NzU1ODYsImhlaWdodCI6MS45MDUwMDA4MDU4NTQ3OTc0fX0seyJwYWdlTnVtYmVyIjo4OSwiYmJveCI6eyJ0b3AiOjY0LjMyNzQ4NDEzMDg1OTM4LCJsZWZ0IjoxNC43MjI4NTM2NjA1ODM0OTYsIndpZHRoIjo1Ny40MDA1MDUwNjU5MTc5NywiaGVpZ2h0IjoxLjkwNTAwMDgwNTg1NDc5NzR9fSx7InBhZ2VOdW1iZXIiOjg5LCJiYm94Ijp7InRvcCI6NjUuOTU1Njg4NDc2NTYyNSwibGVmdCI6MTcuNSwid2lkdGgiOjYzLjA5NTY5OTMxMDMwMjczNCwiaGVpZ2h0IjoxLjkwNTAwMDgwNTg1NDc5NzR9fSx7InBhZ2VOdW1iZXIiOjg5LCJiYm94Ijp7InRvcCI6NjcuNTgzODkyODIyMjY1NjIsImxlZnQiOjE0LjcyMjg1MzY2MDU4MzQ5Niwid2lkdGgiOjY4LjUyMjU5ODI2NjYwMTU2LCJoZWlnaHQiOjEuOTA0OTk2NjMzNTI5NjYzfX0seyJwYWdlTnVtYmVyIjo4OSwiYmJveCI6eyJ0b3AiOjY5LjIxMjA4OTUzODU3NDIyLCJsZWZ0IjoxNC43MjI4NTM2NjA1ODM0OTYsIndpZHRoIjo2NS40MTIyNzcyMjE2Nzk2OSwiaGVpZ2h0IjoxLjkwNTAwMDgwNTg1NDc5NzR9fSx7InBhZ2VOdW1iZXIiOjg5LCJiYm94Ijp7InRvcCI6NzAuODQwMjkzODg0Mjc3MzQsImxlZnQiOjE0LjcyMjg1MzY2MDU4MzQ5Niwid2lkdGgiOjY3Ljc3MDAxOTUzMTI1LCJoZWlnaHQiOjEuOTA1MDA1MDk3Mzg5MjIxMn19LHsicGFnZU51bWJlciI6ODksImJib3giOnsidG9wIjo3Mi40Njg0OTgyMjk5ODA0NywibGVmdCI6MTQuNzIyODUzNjYwNTgzNDk2LCJ3aWR0aCI6NjkuMzg2MzI5NjUwODc4OSwiaGVpZ2h0IjoxLjkwNTAwNTA5NzM4OTIyMTJ9fSx7InBhZ2VOdW1iZXIiOjg5LCJiYm94Ijp7InRvcCI6NzQuMDk2NzAyNTc1NjgzNiwibGVmdCI6MTQuNzIyODUzNjYwNTgzNDk2LCJ3aWR0aCI6MzAuMTM0ODgwMDY1OTE3OTcsImhlaWdodCI6MS45MDUwMDUwOTczODkyMjEyfX0seyJwYWdlTnVtYmVyIjo4OSwiYmJveCI6eyJ0b3AiOjc1LjcyNDkxNDU1MDc4MTI1LCJsZWZ0IjoxNy41LCJ3aWR0aCI6NjQuNDEwNzk3MTE5MTQwNjIsImhlaWdodCI6MS45MDUwMDUwOTczODkyMjEyfX0seyJwYWdlTnVtYmVyIjo4OSwiYmJveCI6eyJ0b3AiOjc3LjM1MzExMTI2NzA4OTg0LCJsZWZ0IjoxNC43MjI4NTM2NjA1ODM0OTYsIndpZHRoIjo2NC44NTU2OTAwMDI0NDE0LCJoZWlnaHQiOjEuOTA1MDA1MDk3Mzg5MjIxMn19LHsicGFnZU51bWJlciI6ODksImJib3giOnsidG9wIjo3OC45ODEzMTU2MTI3OTI5NywibGVmdCI6MTQuNzIyODUzNjYwNTgzNDk2LCJ3aWR0aCI6NjYuMDE5ODUxNjg0NTcwMzEsImhlaWdodCI6MS45MDUwMDUwOTczODkyMjEyfX0seyJwYWdlTnVtYmVyIjo4OSwiYmJveCI6eyJ0b3AiOjgwLjYwOTUyNzU4Nzg5MDYyLCJsZWZ0IjoxNC43MjI4NTM2NjA1ODM0OTYsIndpZHRoIjo2Ny43ODc2NDM0MzI2MTcxOSwiaGVpZ2h0IjoxLjkwNTAwNTA5NzM4OTIyMTJ9fSx7InBhZ2VOdW1iZXIiOjg5LCJiYm94Ijp7InRvcCI6ODIuMjM3NzI0MzA0MTk5MjIsImxlZnQiOjE0LjcyMjg1MzY2MDU4MzQ5Niwid2lkdGgiOjY5LjczOTY5MjY4Nzk4ODI4LCJoZWlnaHQiOjEuOTA1MDA1MDk3Mzg5MjIxMn19LHsicGFnZU51bWJlciI6ODksImJib3giOnsidG9wIjo4My44NjU5MzYyNzkyOTY4OCwibGVmdCI6MTQuNzIyODUzNjYwNTgzNDk2LCJ3aWR0aCI6NjguNDg3MTI5MjExNDI1NzgsImhlaWdodCI6MS45MDUwMDUwOTczODkyMjEyfX1dfSwibG9jYXRvciI6eyJwYWdlTnVtYmVyIjoiOTAifX0seyJvcmlnaW4iOjQsIndvcmsiOnsidHlwZSI6OCwic3VidHlwZSI6eyJvbmVvZktpbmQiOiJhcnRpY2xlVHlwZSIsImFydGljbGVUeXBlIjoxfSwiY29udHJpYnV0b3JzIjpbeyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiRWxpc2EiLCJmYW1pbHkiOiJCZXJkYWxldCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJSYXBoYWVsIE0uIiwiZmFtaWx5IjoiS3VkZWxhIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkVkIiwiZmFtaWx5IjoiVXJiYW4ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiSGVucmlrIiwiZmFtaWx5IjoiRW5ldm9sZHNlbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJOZWlsIFMuIiwiZmFtaWx5IjoiQmFuYXMifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiRWlsZWVuIiwiZmFtaWx5IjoiQnJlc25hbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNaWNoZWxlIEEuIiwiZmFtaWx5IjoiQnVyZm9yZCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLZWl0aCIsImZhbWlseSI6IkRhdmlkc29uIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkNocmlzdG9waGVyIEouIiwiZmFtaWx5IjoiR29ibGVyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkJlbmd0IiwiZmFtaWx5IjoiS2FybHNvbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJQbyBUZWVuIiwiZmFtaWx5IjoiTGltIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkxpbmNvbG4iLCJmYW1pbHkiOiJNYWNLZW56aWUifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTWFyaW5hIiwiZmFtaWx5IjoiTW9udHJlc29yIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlZlcmEgTC4iLCJmYW1pbHkiOiJUcmFpbmVyIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkdpcmVzIiwiZmFtaWx5IjoiVXN1cCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLZWRvbmciLCJmYW1pbHkiOiJZaW4ifX1dLCJpZHMiOnsiaXNzbiI6WyIxMDQyLTgyNzUiLCIyMzc3LTYxN1giXSwiaXNibiI6W10sImRvaSI6IjEwLjU2NzAvb2NlYW5vZy4yMDE3LjExMSIsIm1hZyI6IjI1OTc2NjQ3MjciLCJvcGVuYWxleCI6IlcyNTk3NjY0NzI3In0sInRpdGxlIjoiR2xvYmFsSEFCOiBBIE5ldyBQcm9ncmFtIHRvIFByb21vdGUgSW50ZXJuYXRpb25hbCBSZXNlYXJjaCwgT2JzZXJ2YXRpb25zLCBhbmQgTW9kZWxpbmcgb2YgSGFybWZ1bCBBbGdhbCBCbG9vbXMgaW4gQXF1YXRpYyBTeXN0ZW1zIiwiYWJzdHJhY3QiOiJCZXJkYWxldCwgRWxpc2EgLi4uIGV0IGFsLi0tIFNwZWNpYWwgaXNzdWUgb24gSW50ZXJuYXRpb25hbCBDb29wZXJhdGlvbiBpbiBIYXJtZnVsIEFsZ2FsIEJsb29tIFNjaWVuY2UuLS0gMTIgcGFnZXMsIDYgZmlndXJlcyIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC41NjcwL29jZWFub2cuMjAxNy4xMTEiLCJyZXNvdXJjZVVybCI6Imh0dHBzOi8vdG9zLm9yZy9vY2Vhbm9ncmFwaHkvYXNzZXRzL2RvY3MvMzAtMV9iZXJkYWxldDMucGRmIiwiaXNzdWVkRGF0ZSI6IjIwMTctMDMtMDEiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTEwIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJPY2Vhbm9ncmFwaHkiLCJpc3N1ZSI6MSwidm9sdW1lIjozMCwiaW1wYWN0RmFjdG9yIjoiMi45MyJ9LCJjb250YWluZXJMb2NhdG9yIjp7InBhZ2VSYW5nZSI6eyJmaXJzdFBhZ2UiOjcwLCJsYXN0UGFnZSI6ODF9fSwicHVibGlzaGVySW5mbyI6eyJuYW1lIjoiT2NlYW5vZ3JhcGh5IFNvY2lldHkifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjI2LCJjaXRlZEJ5Q291bnQiOjMyfX0sInF1b3RlIjoiVGhlICBlY29ub21pYyBlZmZlY3RzIG9mIEhBQnMgYXJpc2UgZnJvbSBwdWItbGljIGhlYWx0aCBjb3N0cywgY29tbWVyY2lhbCBmaXNoZXJ5IGFuZCAgYXF1YWN1bHR1cmUgb3BlcmF0aW9uIGNsb3N1cmVzIGFuZCBmaXNoICBraWxscywgcG9zc2libGUgbWVkaXVtLSBhbmQgbG9uZy10ZXJtICBkZWNsaW5lcyBpbiBjb2FzdGFsIGFuZCBtYXJpbmUgcmVjcmVhdGlvbiAgYW5kIHRvdXJpc20sIGFuZCB0aGUgY29zdHMgb2YgaW5zdXJhbmNlLCAgbW9uaXRvcmluZywgbWFuYWdlbWVudCwgYW5kIG1pdGlnYXRpb24gIChNb3JnYW4gZXQgYWwuLCAyMDEwKS4gQWdncmVnYXRpbmcgZWNvLW5vbWljIGVmZmVjdHPigJRib3RoIHdpdGhpbiBhbmQgYWNyb3NzICB0aGVzZSBjYXRlZ29yaWVz4oCUY2FuIGFsc28gYmUgcHJvYmxlbS1hdGljIHdpdGggYXZhaWxhYmxlIGRhdGEgYW5kIGN1cnJlbnRseSB1c2VkICBtZXRob2RvbG9naWVzIChIb2FnbGFuZCBldCBhbC4sIDIwMDIpLiAgQW4gZXhhbXBsZSBvZiBhbiBlY29ub21pYyBldmFsdWF0aW9uICBvZiBIQUJzIGF0IGEgbmF0aW9uYWwgbGV2ZWwgaXMgZ2l2ZW4gYnkgIEFuZGVyc29uIGV0IGFsLiAoMjAwMCksIHdobyBlc3RpbWF0ZWQgdGhlICBlY29ub21pYyBlZmZlY3RzIG9mIEhBQnMgaW4gdGhlIFVuaXRlZCAgU3RhdGVzIHRvIGJlICQxMDAgbWlsbGlvbiBwZXIgeWVhciAoYXMgbGF0ZXIgIHRyYW5zbGF0ZWQgaW50byAyMDEyIGRvbGxhcnMpLCBicm9rZW4gZG93biAgYXMgZm9sbG93czogNDUlIGZvciBwdWJsaWMgaGVhbHRoIGNvc3RzLCAgMzclIGZvciB0aGUgY29zdHMgb2YgY2xvc3VyZXMgYW5kIGxvc3NlcyAgZXhwZXJpZW5jZWQgYnkgY29tbWVyY2lhbCBmaXNoZXJpZXMsICAxMyUgZnJvbSB0aGUgaW1wYWN0IG9uIGxvc3QgcmVjcmVhdGlvbiAgYW5kIHRvdXJpc20gb3Bwb3J0dW5pdGllcywgYW5kIDQlIGZyb20gRklHVVJFIDYuIE1hc3NpdmUgZmlzaCBtb3J0YWxpdGllcyBpbiBKYWthcnRhLCBJbmRvbmVzaWEuIFBob3RvIGNyZWRpdHM6IFR1bXBhayBTaWRhYnV0YXIgKEhBTjUzKSBtb25pdG9yaW5nIGFuZCBtYW5hZ2VtZW50IGNvc3RzLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjExLjgxOTQ0NDY1NjM3MjA3LCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzNDk0NTI5NzI0MTIxLCJoZWlnaHQiOjEuNzAzMjgyNTk0NjgwNzg2MX19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjEzLjUyMzk4OTY3NzQyOTIsImxlZnQiOjY3LjI1NDg5ODA3MTI4OTA2LCJ3aWR0aCI6MjYuODY0Mzc0MTYwNzY2NiwiaGVpZ2h0IjoxLjcwMzI4MjU5NDY4MDc4NjF9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjoxNS4yMjg1MzU2NTIxNjA2NDUsImxlZnQiOjY3LjI1NDg5ODA3MTI4OTA2LCJ3aWR0aCI6MjcuMjMyMDAyMjU4MzAwNzgsImhlaWdodCI6MS43MDMyODM1NDgzNTUxMDI1fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6MTYuOTMzMDgwNjczMjE3NzczLCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzMzYzNjg1NjA3OTEsImhlaWdodCI6MS43MDMyODM1NDgzNTUxMDI1fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6MTguNjM3NjI2NjQ3OTQ5MjIsImxlZnQiOjY3LjI1NDg5ODA3MTI4OTA2LCJ3aWR0aCI6MjcuMjM1Mjk0MzQyMDQxMDE2LCJoZWlnaHQiOjEuNzAzMjgzNTQ4MzU1MTAyNX19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjIwLjM0MjE3MDcxNTMzMjAzLCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzMzk4NTkwMDg3ODkwNiwiaGVpZ2h0IjoxLjcwMzI4MzU0ODM1NTEwMjV9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjoyMi4wNDY3MTY2OTAwNjM0NzcsImxlZnQiOjY3LjI1NDg5ODA3MTI4OTA2LCJ3aWR0aCI6MjcuMjMyMDExNzk1MDQzOTQ1LCJoZWlnaHQiOjEuNzAzMjgzNTQ4MzU1MTAyNX19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjIzLjc1MTI2MjY2NDc5NDkyMiwibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzIwNDIzMTI2MjIwNywiaGVpZ2h0IjoxLjcwMzI4MzU0ODM1NTEwMjV9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjoyNS40NTU4MDg2Mzk1MjYzNjcsImxlZnQiOjY3LjI1NDg5ODA3MTI4OTA2LCJ3aWR0aCI6MjYuODYzMDY3NjI2OTUzMTI1LCJoZWlnaHQiOjEuNzAzMjgzNTQ4MzU1MTAyNX19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjI3LjE2MDM1MjcwNjkwOTE4LCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzNTQzMzU3ODQ5MTIxLCJoZWlnaHQiOjEuNzAzMjgzNTQ4MzU1MTAyNX19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjI4Ljg2NDg5ODY4MTY0MDYyNSwibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNi44NjQzOTUxNDE2MDE1NjIsImhlaWdodCI6MS43MDMyODM1NDgzNTUxMDI1fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6MzAuNTY5NDQyNzQ5MDIzNDM4LCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzNDkyNDMxNjQwNjI1LCJoZWlnaHQiOjEuNzAzMjgzNTQ4MzU1MTAyNX19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjMyLjI3Mzk5MDYzMTEwMzUxNiwibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzQ3NzU1NDMyMTI4OSwiaGVpZ2h0IjoxLjcwMzI4MzU0ODM1NTEwMjV9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjozMy45Nzg1MzQ2OTg0ODYzMywibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzQzMzY4NTMwMjczNDQsImhlaWdodCI6MS43MDMyODU0NTU3MDM3MzU0fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6MzUuNjgzMDc4NzY1ODY5MTQsImxlZnQiOjY3LjI1NDg5ODA3MTI4OTA2LCJ3aWR0aCI6MjcuMjMyMTYwNTY4MjM3MzA1LCJoZWlnaHQiOjEuNzAzMjg1NDU1NzAzNzM1NH19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjM3LjM4NzYyMjgzMzI1MTk1LCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzMzY2NTQ2NjMwODU5NCwiaGVpZ2h0IjoxLjcwMzI4NTQ1NTcwMzczNTR9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjozOS4wOTIxNzA3MTUzMzIwMywibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzY1Nzk4OTUwMTk1MywiaGVpZ2h0IjoxLjcwMzI4NTQ1NTcwMzczNTR9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo0MC43OTY3MTQ3ODI3MTQ4NDQsImxlZnQiOjY3LjI1NDg5ODA3MTI4OTA2LCJ3aWR0aCI6MjcuMjM2NzE3MjI0MTIxMDk0LCJoZWlnaHQiOjEuNzAzMjg1NDU1NzAzNzM1NH19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjQyLjUwMTI2MjY2NDc5NDkyLCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzMzYyNzMxOTMzNTkzOCwiaGVpZ2h0IjoxLjcwMzI4NTQ1NTcwMzczNTR9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo0NC4yMDU4MDY3MzIxNzc3MzQsImxlZnQiOjY3LjI1NDg5ODA3MTI4OTA2LCJ3aWR0aCI6MjcuMjMwNTQ1MDQzOTQ1MzEyLCJoZWlnaHQiOjEuNzAzMjg1NDU1NzAzNzM1NH19LHsicGFnZU51bWJlciI6OCwiYmJveCI6eyJ0b3AiOjQ1LjkxMDM1MDc5OTU2MDU1LCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzODU0NDQ2NDExMTMyOCwiaGVpZ2h0IjoxLjcwMzI4NTQ1NTcwMzczNTR9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo0Ny42MTQ4OTQ4NjY5NDMzNiwibGVmdCI6NjcuMjU0ODk4MDcxMjg5MDYsIndpZHRoIjoyNy4yMzQxNDYxMTgxNjQwNjIsImhlaWdodCI6MS43MDMyODU0NTU3MDM3MzU0fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6NDkuMzE5NDQyNzQ5MDIzNDQsImxlZnQiOjY3LjI1NDg5ODA3MTI4OTA2LCJ3aWR0aCI6MjcuMjM1OTQwOTMzMjI3NTQsImhlaWdodCI6MS43MDMyODU0NTU3MDM3MzU0fX0seyJwYWdlTnVtYmVyIjo4LCJiYm94Ijp7InRvcCI6NTEuMDIzOTkwNjMxMTAzNTE2LCJsZWZ0Ijo2Ny4yNTQ4OTgwNzEyODkwNiwid2lkdGgiOjI3LjIzNjkwNzk1ODk4NDM3NSwiaGVpZ2h0IjoxLjcwMzI4NTQ1NTcwMzczNTR9fSx7InBhZ2VOdW1iZXIiOjgsImJib3giOnsidG9wIjo5Mi4yNzg3NzA0NDY3NzczNCwibGVmdCI6OC44MjM1MjkyNDM0NjkyMzgsIndpZHRoIjo1Ni4yNjM5MTYwMTU2MjUsImhlaWdodCI6MS4yMDkwODk1MTc1OTMzODM4fX0seyJwYWdlTnVtYmVyIjo5LCJiYm94Ijp7InRvcCI6NS4wMDEyNjI2NjQ3OTQ5MjIsImxlZnQiOjUuODgyMzUyODI4OTc5NDkyLCJ3aWR0aCI6MjcuMjM1Mjk0MzQyMDQxMDE2LCJoZWlnaHQiOjEuNzAzMjgyNTk0NjgwNzg2MX19XX0sImxvY2F0b3IiOnsicGFnZU51bWJlciI6Ijc4In19XQ%3D%3D. Challenges like youth disinterest are met by gamified apps digitizing practices (Paerl et al., 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo0LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkhhbnMgVy4iLCJmYW1pbHkiOiJQYWVybCJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJLLkUuIiwiZmFtaWx5IjoiSGF2ZW5zIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6Ik4uUy4iLCJmYW1pbHkiOiJIYWxsIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlQuRy4iLCJmYW1pbHkiOiJPdHRlbiJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJNLiIsImZhbWlseSI6IlpodSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJIYWlodWkiLCJmYW1pbHkiOiJYdSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJHLiIsImZhbWlseSI6IlpodSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJCLiIsImZhbWlseSI6IlFpbiJ9fV0sImlkcyI6eyJpc3NuIjpbXSwiaXNibiI6W10sImRvaSI6IjEwLjE3NjE1LzRua2Qtems5MyIsIm9wZW5hbGV4IjoiVzQzMDcwNjI0MjkifSwidGl0bGUiOiJNaXRpZ2F0aW5nIGEgZ2xvYmFsIGV4cGFuc2lvbiBvZiB0b3hpYyBjeWFub2JhY3RlcmlhbCBibG9vbXM6IENvbmZvdW5kaW5nIGVmZmVjdHMgYW5kIGNoYWxsZW5nZXMgcG9zZWQgYnkgY2xpbWF0ZSBjaGFuZ2UiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTc2MTUvNG5rZC16azkzIiwicmVzb3VyY2VVcmwiOiJodHRwczovL2Nkci5saWIudW5jLmVkdS9kb3dubG9hZHMvMDU3NDIyODM0IiwiaXNzdWVkRGF0ZSI6IjIwMjAtMDEtMDEiLCJhY2Nlc3NlZERhdGUiOiIyMDI1LTAxIiwiY29udGFpbmVySW5mbyI6eyJhdXRob3JzIjpbXSwidGl0bGUiOiJDYXJvbGluYSBEaWdpdGFsIFJlcG9zaXRvcnkgKFVuaXZlcnNpdHkgb2YgTm9ydGggQ2Fyb2xpbmEgYXQgQ2hhcGVsIEhpbGwpIn0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IlVuaXZlcnNpdHkgb2YgTm9ydGggQ2Fyb2xpbmEgYXQgQ2hhcGVsIEhpbGwifSwidGFsbHkiOnsicmVmZXJlbmNlZFdvcmtzQ291bnQiOjAsImNpdGVkQnlDb3VudCI6MH19LCJxdW90ZSI6IlRoYXQgcmVsYXRpb25zaGlwIGhhcyBzZXJ2ZWQgYXMgdGhlIGZvdW5kYXRpb24gZm9yIHNldHRpbmcgIG51dHJpZW50IGxvYWRpbmcgdGFyZ2V0cyBpbiBsYWtlcyBhcm91bmQgdGhlIHdvcmxkLCBhbmQgY2FyZWZ1bGx5ICBkZXNpZ25lZCBleHBlcmltZW50cyBtYXkgZWx1Y2lkYXRlIHRvIHdoYXQgZXh0ZW50IHRlbXBlcmF0dXJlICBpbmNyZWFzZSBhZmZlY3RzIHRoZSB0aHJlc2hvbGQgYXQgd2hpY2ggdGhlIGZyZXF1ZW5jeSBvZiBvY2N1ciAgcmVuY2Ugb2YgQ3lhbm9IQUJzIHF1aWNrbHkgcmlzZXMgd2l0aCBpbmNyZWFzaW5nIE4gYW5kIFAuIENvbmNsdXNpb25zIFJlY2VudCBzdHVkaWVzIGluZGljYXRlIHRoYXQgdGhlIHN5bmVyZ2lzdGljIGNvbWJpbmF0aW9uIG9mIGFudGhyb3BvZ2VuaWMgbnV0cmllbnQgbG9hZGluZywgcmlzaW5nIHRlbXBlcmF0dXJlcywgZW5oYW5jZWQgIHZlcnRpY2FsIHN0cmF0aWZpY2F0aW9uLCBpbmNyZWFzZWQgcmVzaWRlbmNlIHRpbWUgYW5kIG1vcmUgIGV4dHJlbWUgY2xpbWF0aWMgY29uZGl0aW9ucyBvdmVyYWxsIHdpbGwgZmF2b3VyIGN5YW5vYmFjdGVyaWFsICBkb21pbmFuY2UgaW4gYSB3aWRlIHJhbmdlIG9mIGFxdWF0aWMgZWNvc3lzdGVtcy4gVGhlIGV4cGFuICBzaW9uIG9mIEN5YW5vSEFCcyBoYXMgc2VyaW91cyBjb25zZXF1ZW5jZXMgZm9yIGh1bWFuIHdhdGVyICBzdXBwbGllcywgZmlzaGVyaWVzIGFuZCByZWNyZWF0aW9uYWwgcmVzb3VyY2VzLiBFY29zeXN0ZW0gc2NhbGUgIHBoeXNpY2FsIGNoZW1pY2FsIGJpb3RpYyBtaXRpZ2F0aW9uIHN0cmF0ZWdpZXMgd2lsbCBoYXZlIHRvICBpbmNvcnBvcmF0ZSBudXRyaWVudCAobW9zdCBvZnRlbiBib3RoIE4gYW5kIFApIGlucHV0IHJlZHVjdGlvbnMuICBUaGUgbWFnbml0dWRlcyBvZiB0aGVzZSByZWR1Y3Rpb25zIHdpbGwgbmVlZCB0byBiZSBzeXN0ZW0gIHNwZWNpZmljIGFuZCB3aWxsIGxpa2VseSBuZWVkIHRvIGJlIGFkanVzdGVkIHdpdGggY2hhbmdpbmcgY2xpICBtYXRpYyBjb25kaXRpb25zLiBTcGVjaWZpY2FsbHksIHdhdGVyIHF1YWxpdHkgbWFuYWdlcnMgd2lsbCBoYXZlICB0byBhY2NvbW1vZGF0ZSB3YXJtaW5nIGFuZCBncmVhdGVyIGh5ZHJvbG9naWNhbCB2YXJpYWJpbGl0eSAgYW5kIGV4dHJlbWVuZXNzIHJlc3VsdGluZyBmcm9tIGNsaW1hdGUgY2hhbmdlIGluIGZvcm11bGF0aW5nICB0aGVpciBzdHJhdGVnaWVzLiBBIGtleSBsb25nIHRlcm0gY29udHJvbCB3ZSBjYW4gZXhlcnQgdG8gcmVkdWNlICB0aGUgcmF0ZSBhbmQgZXh0ZW50IG9mIGdsb2JhbCB3YXJtaW5nIGlzIGN1cmJpbmcgZ3JlZW5ob3VzZSBnYXMgIGVtaXNzaW9ucy4gV2l0aG91dCB0aGlzIGVzc2VudGlhbCBzdGVwLCBmdXR1cmUgd2FybWluZyB0cmVuZHMsICBoeWRyb2xvZ2ljYWwgZXh0cmVtZXMgKHJlY29yZCByYWluZmFsbHMgYW5kIGRyb3VnaHRzKSBhbmQgdGhlaXIgIGVmZmVjdHMgb24gYXF1YXRpYyBlY29zeXN0ZW1zIHdpbGwgY29udGludWUgdG8gcHJvbW90ZSBvcHBvciAgdHVuaXN0aWMgQ3lhbm9IQUJzLiAiLCJwZGZIaWdobGlnaHRzIjp7ImxvY2F0aW9ucyI6W3sicGFnZU51bWJlciI6MTAsImJib3giOnsidG9wIjoxNC4xNDkyMzM4MTgwNTQyLCJsZWZ0IjoxMy4xMjg1OTUzNTIxNzI4NTIsIndpZHRoIjozNy4wODIxNzIzOTM3OTg4MywiaGVpZ2h0IjoxLjUxNjYwMjI3Nzc1NTczNzN9fSx7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6MTUuNDUxNTA4NTIyMDMzNjkxLCJsZWZ0IjoxMy4xMjg1NDQ4MDc0MzQwODIsIndpZHRoIjozNy4wODQ4MzEyMzc3OTI5NywiaGVpZ2h0IjoxLjUxNjYwMzIzMTQzMDA1Mzd9fSx7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6MTYuNzUzNzg0MTc5Njg3NSwibGVmdCI6MTMuMTI4NTQyOTAwMDg1NDUsIndpZHRoIjozNy4wODMyMDk5OTE0NTUwOCwiaGVpZ2h0IjoxLjUxNjYwNDE4NTEwNDM3MDF9fSx7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6MTguMDU2MDUzMTYxNjIxMDk0LCJsZWZ0IjoxMy4xMjg1NDQ4MDc0MzQwODIsIndpZHRoIjozNi42MTUwNzQxNTc3MTQ4NDQsImhlaWdodCI6MS41MTY2MDQxODUxMDQzNzAxfX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjE5LjQ2ODM3ODA2NzAxNjYsImxlZnQiOjEzLjEyODU0NDgwNzQzNDA4Miwid2lkdGgiOjM0LjI5MDUzMTE1ODQ0NzI2NiwiaGVpZ2h0IjoxLjI0NTAxMTIxMDQ0MTU4OTR9fSx7InBhZ2VOdW1iZXIiOjEwLCJiYm94Ijp7InRvcCI6MjIuMjI2MTQyODgzMzAwNzgsImxlZnQiOjEzLjEyODU1MDUyOTQ3OTk4LCJ3aWR0aCI6Ny42NDk4NzM3MzM1MjA1MDgsImhlaWdodCI6MS4xMzU0ODMxNDU3MTM4MDYyfX0seyJwYWdlTnVtYmVyIjoxMCwiYmJveCI6eyJ0b3AiOjIzLjczNDg0OTkyOTgwOTU3LCJsZWZ0IjoxMy4xMjg1MzUyNzA2OTA5MTgsIndpZHRoIjozNy4wODkyODY4MDQxOTkyMiwiaGVpZ2h0IjoyNi4wODg2NDAyMTMwMTI2OTV9fV19LCJsb2NhdG9yIjp7InBhZ2VOdW1iZXIiOiIxMSJ9fV0%3D.

Hybridizing these NBS with citizen science yields transformative outcomes: granular monitoring exposes gaps, while TEK ensures cultural fit. Scaling via state wetland authorities promises resilient lakes, blending bottom-up action with policy for sustained HAB mitigation (Sampat et al., 2020)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFwb29ydmEgTS4iLCJmYW1pbHkiOiJTYW1wYXQifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW5kcmVhIiwiZmFtaWx5IjoiSGlja3MifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiR2VyYXJkbyBKLiIsImZhbWlseSI6IlJ1aXrigJBNZXJjYWRvIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IlbDrcKtY3RvciBNLiIsImZhbWlseSI6IlphdmFsYSJ9fV0sImlkcyI6eyJpc3NuIjpbIjA5MjEtMzQ0OSIsIjE4NzktMDY1OCJdLCJpc2JuIjpbXSwiZG9pIjoiMTAuMTAxNi9qLnJlc2NvbnJlYy4yMDIwLjEwNTE5OSIsIm1hZyI6IjMwOTQ2MjUxMDQiLCJwbWlkIjoiMzM3NDYzNjEiLCJvcGVuYWxleCI6IlczMDk0NjI1MTA0In0sInRpdGxlIjoiVmFsdWluZyBlY29ub21pYyBpbXBhY3QgcmVkdWN0aW9ucyBvZiBudXRyaWVudCBwb2xsdXRpb24gZnJvbSBsaXZlc3RvY2sgd2FzdGUiLCJ1cmwiOiJodHRwczovL2RvaS5vcmcvMTAuMTAxNi9qLnJlc2NvbnJlYy4yMDIwLjEwNTE5OSIsInJlc291cmNlVXJsIjoiaHR0cHM6Ly93d3cubmNiaS5ubG0ubmloLmdvdi9wbWMvYXJ0aWNsZXMvNzk3MDUwNSIsImlzc3VlZERhdGUiOiIyMDIwLTEwLTE1IiwiYWNjZXNzZWREYXRlIjoiMjAyNi0wMSIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W10sInRpdGxlIjoiUmVzb3VyY2VzIENvbnNlcnZhdGlvbiBhbmQgUmVjeWNsaW5nIiwidm9sdW1lIjoxNjQsImltcGFjdEZhY3RvciI6IjExLjAxIn0sImNvbnRhaW5lckxvY2F0b3IiOnsicGFnZVJhbmdlIjp7ImZpcnN0UGFnZSI6MTA1MTk5LCJsYXN0UGFnZSI6MTA1MTk5fX0sInB1Ymxpc2hlckluZm8iOnsibmFtZSI6IkVsc2V2aWVyIEJWIn0sInRhbGx5Ijp7InJlZmVyZW5jZWRXb3Jrc0NvdW50IjozOSwiY2l0ZWRCeUNvdW50Ijo1N319LCJxdW90ZSI6IlNlY3Rpb24gVGl0bGU6IFZhbHVpbmcgZWNvbm9taWMgaW1wYWN0IHJlZHVjdGlvbnMgb2YgbnV0cmllbnQgcG9sbHV0aW9uIGZyb20gbGl2ZXN0b2NrIHdhc3RlID4gMS4gSW50cm9kdWN0aW9uXG5Db250ZW50OlxuSW4gdGhlIFN0YXRlIG9mIEZsb3JpZGEsIEhBQnMgaGF2ZSByZXN1bHRlZCBpbiBhIG1vbnRobHkgbG9zcyBvZiAyLjggYW5kIDMuNyBtaWxsaW9uIFVTRCBjb3JyZXNwb25kaW5nIHRvIHJlc3RhdXJhbnQgYW5kIGxvZGdpbmcgcmV2ZW51ZSwgcmVzcGVjdGl2ZWx5ICggW0xhcmtpbiBhbmQgQWRhbXMsIDIwMDddKCkgKS4gRnJlcXVlbnQgb2NjdXJyZW5jZSBvZiBIQUJzIGFsc28gbG93ZXJzIHByb3BlcnR5IHZhbHVlcyBvZiBsYWtlZnJvbnQgcHJvcGVydGllcy4gVGhlIGxvc3MgaW4gcHJvcGVydHkgdmFsdWVzIGFyZSB0aGUgbGFyZ2VzdCBpbXBhY3QgYmVhcmVycyBvZiBldXRyb3BoaWNhdGlvbiB3aXRoIGFuIGVzdGltYXRlZCBhdmVyYWdlIGVjb25vbWljIGxvc3Mgb2YgMS42IGJpbGxpb24gVVNEIGFubnVhbGx5ICggW0RvZGRzIGV0IGFsLiwgMjAwOV0oKSApLiJ9XQ%3D%3D. The main community-level strategies for HAB mitigation and lake restoration are summarized in Table 6. Establishing incentive programs for local communities to adopt these practices further strengthens implementation, while enhanced monitoring frameworks allow for the measurement of long-term impacts on water quality (Ronoh et al., 2025)https://dummy-citation.com/citation?d=W3sib3JpZ2luIjo1LCJ3b3JrIjp7InR5cGUiOjgsInN1YnR5cGUiOnsib25lb2ZLaW5kIjoiYXJ0aWNsZVR5cGUiLCJhcnRpY2xlVHlwZSI6MX0sImNvbnRyaWJ1dG9ycyI6W3sicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkVybmVzdCIsImZhbWlseSI6IlJvbm9oIn19LHsicm9sZSI6MCwibmFtZSI6eyJnaXZlbiI6IkFubmlrYSIsImZhbWlseSI6IlNjaGxlbW0ifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiRXJhc3RvIEJlbmVkaWN0IiwiZmFtaWx5IjoiTWthbWEifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiRG91Z2xhcyIsImZhbWlseSI6Ik55b2xlaSJ9fSx7InJvbGUiOjAsIm5hbWUiOnsiZ2l2ZW4iOiJKb2huIE1haW5hIiwiZmFtaWx5IjoiTnlvbmdlc2EifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiTWF1cmljZSIsImZhbWlseSI6Ik55YWRhd2EifX0seyJyb2xlIjowLCJuYW1lIjp7ImdpdmVuIjoiQW5uIHZhbiIsImZhbWlseSI6IkdyaWVuc3ZlbiJ9fV0sImlkcyI6eyJpc3NuIjpbXSwiaXNibiI6W10sImRvaSI6IjEwLjUxOTQvZWd1c3BoZXJlLWVndTI1LTE4MjU5Iiwib3BlbmFsZXgiOiJXNDQwODQ4NTY4NCIsImV4YSI6Imh0dHBzOi8vd3d3LnNjaWxpdC5jb20vcHVibGljYXRpb25zLzY4YjY1MTBlMGRhYTkzNTgxNTRjNzk5MWNjZjAzNWVhIn0sInRpdGxlIjoiQXBwbGljYXRpb24gb2YgTmF0dXJlLUJhc2VkIFNvbHV0aW9ucyAoTmJTKSBmb3IgQ2F0Y2htZW50IFdhdGVyIFF1YWxpdHkgTWFuYWdlbWVudCBpbiBZYWxhIFJpdmVyIEJhc2luLCBXZXN0ZXJuIEtlbnlhLiIsImFic3RyYWN0IjoiTmF0dXJlLWJhc2VkIHNvbHV0aW9ucyAoTmJTKSBoYXZlIGVtZXJnZWQgYXMgaW5ub3ZhdGl2ZSBhbmQgc3VzdGFpbmFibGUgYXBwcm9hY2hlcyB0byBhZGRyZXNzIGVudmlyb25tZW50YWwgY2hhbGxlbmdlcywgaW5jbHVkaW5nIHdhdGVyIHF1YWxpdHkgZGVncmFkYXRpb24gaW4gcml2ZXIgY2F0Y2htZW50cy4gVGhlc2Ugc29sdXRpb25zIGxldmVyYWdlIG5hdHVyYWwgcHJvY2Vzc2VzIGFuZCBlY29zeXN0ZW1zIHRvIGVuaGFuY2Ugd2F0ZXIgcXVhbGl0eSB3aGlsZSBwcm92aWRpbmcgY28tYmVuZWZpdHMgc3VjaCBhcyBiaW9kaXZlcnNpdHkgY29uc2VydmF0aW9uIGFuZCBjbGltYXRlIHJlc2lsaWVuY2UuIFRoZSBZYWxhIFJpdmVyIEJhc2luIGluIEtlbnlhIGZhY2VzIHNpZ25pZmljYW50IHdhdGVyIHF1YWxpdHkgY2hhbGxlbmdlcyBkdWUgdG8gaW5jcmVhc2VkIHNlZGltZW50IHlpZWxkcyBhbmQgbnV0cmllbnQgbG9hZGluZywgcHJpbWFyaWx5IGRyaXZlbiBieSBhZ3JpY3VsdHVyYWwgYWN0aXZpdGllcywgZGVmb3Jlc3RhdGlvbiwgYW5kIGxhbmQtdXNlIGNoYW5nZXMuIFRoaXMgc3R1ZHkgZXhwbG9yZXMgdGhlIGFwcGxpY2F0aW9uIG9mIE5iUywgc3VjaCBhcyByaXBhcmlhbiBidWZmZXIgc3RyaXBzLCBhZ3JvZm9yZXN0cnksIGFuZCB3ZXRsYW5kIHJlc3RvcmF0aW9uLCBpbiBtaXRpZ2F0aW5nIHdhdGVyIHF1YWxpdHkgaXNzdWVzIGluIHRoZSBZYWxhIEJhc2luLiBUaGUgU29pbCBhbmQgV2F0ZXIgQXNzZXNzbWVudCBUb29sIFBsdXMgKFNXQVQrKSB3YXMgZW1wbG95ZWQgdG8gbW9kZWwgdGhlIGltcGFjdHMgb2YgTmJTIG9uIHdhdGVyIHF1YWxpdHkgaW4gdGhlIFlhbGEgQmFzaW4uIFNXQVQrIGlzIGEgcm9idXN0LCBwcm9jZXNzLWJhc2VkIGh5ZHJvbG9naWNhbCBtb2RlbCB0aGF0IHNpbXVsYXRlcyBsYW5kIHVzZSwgY2xpbWF0ZSwgYW5kIG1hbmFnZW1lbnQgaW50ZXJ2ZW50aW9ucyBhdCB0aGUgY2F0Y2htZW50IHNjYWxlLiBDbGltYXRlIGRhdGEgZnJvbSBJU0lNSVAzYiAoSW50ZXItU2VjdG9yYWwgSW1wYWN0IE1vZGVsIEludGVyY29tcGFyaXNvbiBQcm9qZWN0KSB3ZXJlIHVzZWQgdG8gcHJvdmlkZSBoaWdoLXJlc29sdXRpb24gcHJvamVjdGlvbnMgb2YgY2xpbWF0ZSB2YXJpYWJpbGl0eSBhbmQgY2hhbmdlLiBCYXNlbGluZSBsYW5kLXVzZSBkYXRhIHdlcmUgZGVyaXZlZCBmcm9tIHJlbW90ZSBzZW5zaW5nIGFuZCB2YWxpZGF0ZWQgdXNpbmcgZ3JvdW5kIHN1cnZleXMuIEh5ZHJvbG9naWNhbCByZXNwb25zZSB1bml0cyAoSFJVcykgd2VyZSBkZWZpbmVkIHRvIGNhcHR1cmUgc3BhdGlhbCBoZXRlcm9nZW5laXR5IGluIGxhbmQgY292ZXIsIHNvaWwsIGFuZCB0b3BvZ3JhcGh5LiBTY2VuYXJpb3Mgd2l0aCBhbmQgd2l0aG91dCBOYlMgaW50ZXJ2ZW50aW9ucyB3ZXJlIHNpbXVsYXRlZCB0byBhc3Nlc3MgdGhlaXIgaW1wYWN0IG9uIHNlZGltZW50IGFuZCBudXRyaWVudCB5aWVsZHMuUHJlbGltaW5hcnkgcmVzdWx0cyBpbmRpY2F0ZSB0aGF0IHRoZSBpbXBsZW1lbnRhdGlvbiBvZiBOYlMgc2lnbmlmaWNhbnRseSByZWR1Y2VzIHNlZGltZW50IHlpZWxkcyBhbmQgbnV0cmllbnQgY29uY2VudHJhdGlvbnMgaW4gdGhlIFlhbGEgQmFzaW4uIFNwZWNpZmljYWxseSwgaHlkcm9sb2dpY2FsIHJlc3BvbnNlIHVuaXRzIHdpdGggTmJTIGludGVydmVudGlvbnMgZGVtb25zdHJhdGVkOiBSZWR1Y2VkIHNlZGltZW50IHlpZWxkczogQXJlYXMgd2l0aCByaXBhcmlhbiBidWZmZXIgc3RyaXBzIGFuZCByZXN0b3JlZCB3ZXRsYW5kcyBzaG93ZWQgdXAgdG8gYSAxNSUgcmVkdWN0aW9uIGluIHNlZGltZW50IHRyYW5zcG9ydCBjb21wYXJlZCB0byBiYXNlbGluZSBzY2VuYXJpb3MuIERlY3JlYXNlZCBudXRyaWVudCBsb2FkczogQWdyb2ZvcmVzdHJ5IHByYWN0aWNlcyBhbmQgdmVnZXRhdGlvbiBidWZmZXJzIHJlZHVjZWQgbml0cm9nZW4gYW5kIHBob3NwaG9ydXMgcnVub2ZmIGJ5IGFwcHJveGltYXRlbHkgNCUgYW5kIDclLCByZXNwZWN0aXZlbHkuIFRoZXNlIHJlZHVjdGlvbnMgd2VyZSBwYXJ0aWN1bGFybHkgcHJvbm91bmNlZCBkdXJpbmcgcGVhayByYWluZmFsbCBldmVudHMsIGRlbW9uc3RyYXRpbmcgdGhlIGVmZmVjdGl2ZW5lc3Mgb2YgTmJTIGluIG1pdGlnYXRpbmcgcnVub2ZmLXJlbGF0ZWQgcG9sbHV0aW9uLlRoZSBhcHBsaWNhdGlvbiBvZiBOYlMgaW4gdGhlIFlhbGEgQmFzaW4gZGVtb25zdHJhdGVzIHRoZWlyIHBvdGVudGlhbCB0byBzaWduaWZpY2FudGx5IGltcHJvdmUgY2F0Y2htZW50IHdhdGVyIHF1YWxpdHkgd2hpbGUgZGVsaXZlcmluZyBhbmNpbGxhcnkgZWNvc3lzdGVtIHNlcnZpY2VzLiBTV0FUKyBtb2RlbGluZyBoaWdobGlnaHRzIHRoZSBhYmlsaXR5IG9mIE5iUyB0byBhZGRyZXNzIHNlZGltZW50IGFuZCBudXRyaWVudC1yZWxhdGVkIGNoYWxsZW5nZXMgZWZmZWN0aXZlbHksIGV2ZW4gdW5kZXIgdmFyeWluZyBjbGltYXRpYyBjb25kaXRpb25zLiBUaGVzZSBzb2x1dGlvbnMgbm90IG9ubHkgcHJvdmlkZSBpbW1lZGlhdGUgd2F0ZXIgcXVhbGl0eSBiZW5lZml0cyBidXQgYWxzbyBjb250cmlidXRlIHRvIGxvbmctdGVybSBjYXRjaG1lbnQgcmVzaWxpZW5jZSB0byBjbGltYXRlIGNoYW5nZSBhbmQgYW50aHJvcG9nZW5pYyBwcmVzc3VyZXMuQmFzZWQgb24gdGhlIGZpbmRpbmdzLCBpdCBpcyByZWNvbW1lbmRlZCB0aGF0IHBvbGljeW1ha2VycyBhbmQgc3Rha2Vob2xkZXJzIHByaW9yaXRpemUgdGhlIGludGVncmF0aW9uIG9mIE5iUyBpbiBjYXRjaG1lbnQgbWFuYWdlbWVudCBwbGFucy4gS2V5IHJlY29tbWVuZGF0aW9ucyBpbmNsdWRlOiBTY2FsaW5nIHVwIHJpcGFyaWFuIGJ1ZmZlciB6b25lcyBhbmQgYWdyb2ZvcmVzdHJ5IHN5c3RlbXMgaW4gY3JpdGljYWwgaHlkcm9sb2dpY2FsIHJlc3BvbnNlIHVuaXRzLiBFc3RhYmxpc2hpbmcgaW5jZW50aXZlIHByb2dyYW1zIGZvciBsb2NhbCBjb21tdW5pdGllcyB0byBhZG9wdCBOYlMgcHJhY3RpY2VzLiBFbmhhbmNpbmcgbW9uaXRvcmluZyBhbmQgZXZhbHVhdGlvbiBmcmFtZXdvcmtzIHRvIG1lYXN1cmUgdGhlIGxvbmctdGVybSBpbXBhY3RzIG9mIE5iUyBvbiB3YXRlciBxdWFsaXR5LiBTdHJlbmd0aGVuaW5nIHBhcnRuZXJzaGlwcyBiZXR3ZWVuIGdvdmVybm1lbnQgYWdlbmNpZXMsIHJlc2VhcmNoIGluc3RpdHV0aW9ucywgYW5kIGNvbW11bml0eSBvcmdhbml6YXRpb25zIHRvIHByb21vdGUgdGhlIGNvLWRlc2lnbiBhbmQgaW1wbGVtZW50YXRpb24gb2YgTmJTLiIsInVybCI6Imh0dHBzOi8vZG9pLm9yZy8xMC41MTk0L2VndXNwaGVyZS1lZ3UyNS0xODI1OSIsImlzc3VlZERhdGUiOiIyMDI1LTAzLTE1IiwiYWNjZXNzZWREYXRlIjoiMjAyNS0xMSIsImNvbnRhaW5lckluZm8iOnsiYXV0aG9ycyI6W119LCJ0YWxseSI6eyJyZWZlcmVuY2VkV29ya3NDb3VudCI6MCwiY2l0ZWRCeUNvdW50IjowfX0sInF1b3RlIjoiS2V5IHJlY29tbWVuZGF0aW9ucyBpbmNsdWRlOiBTY2FsaW5nIHVwIHJpcGFyaWFuIGJ1ZmZlciB6b25lcyBhbmQgYWdyb2ZvcmVzdHJ5IHN5c3RlbXMgaW4gY3JpdGljYWwgaHlkcm9sb2dpY2FsIHJlc3BvbnNlIHVuaXRzLiBFc3RhYmxpc2hpbmcgaW5jZW50aXZlIHByb2dyYW1zIGZvciBsb2NhbCBjb21tdW5pdGllcyB0byBhZG9wdCBOYlMgcHJhY3RpY2VzLiBFbmhhbmNpbmcgbW9uaXRvcmluZyBhbmQgZXZhbHVhdGlvbiBmcmFtZXdvcmtzIHRvIG1lYXN1cmUgdGhlIGxvbmctdGVybSBpbXBhY3RzIG9mIE5iUyBvbiB3YXRlciBxdWFsaXR5LiBTdHJlbmd0aGVuaW5nIHBhcnRuZXJzaGlwcyBiZXR3ZWVuIGdvdmVybm1lbnQgYWdlbmNpZXMsIHJlc2VhcmNoIGluc3RpdHV0aW9ucywgYW5kIGNvbW11bml0eSBvcmdhbml6YXRpb25zIHRvIHByb21vdGUgdGhlIGNvLWRlc2lnbiBhbmQgaW1wbGVtZW50YXRpb24gb2YgTmJTLiIsImV4dGVybmFsQ2FjaGVJZCI6IjE1NWI5MWIwLWU3ODUtMzBlYi04YThmLWE3YWQ3MTk5YmQ2OSJ9XQ%3D%3D.

Table 6: Community-Level Management Strategies

| Strategy | Description |
| --- | --- |
| Citizen Science | Empowering local communities to monitor water quality and report early warning signs |
| Traditional Ecological Knowledge | Reviving indigenous water management practices for sustainable stewardship |
| Nature-Based Solutions | Implementing floating wetlands, riparian buffers, and localized waste management |
| Hybrid Approaches | Integrating bottom-up initiatives with top-down policies for resilient governance |

Conclusion

The escalating crisis of Harmful Algal Blooms in Indian lakes is a complex manifestation of nutrient pollution, climate change, and ecological neglect. As species like Microcystis aeruginosa continue to proliferate, the resulting hypoxia and cyanotoxin releases threaten aquatic biodiversity, public health, and local economies. While large-scale regulatory frameworks are essential, sustainable remediation ultimately relies on robust, community-level management. By integrating indigenous ecological knowledge with modern citizen science, continuous water quality monitoring, and localized bioremediation techniques like floating treatment wetlands, communities can actively restore their local water bodies. Protecting India's fragile lake ecosystems requires a decentralized, participatory approach, ensuring that local stakeholders are empowered to combat eutrophication, preserve freshwater resources, and secure a resilient environmental future for generations to come.

References

- Berdalet, E., Kudela, R. M., Urban, E., Enevoldsen, H., Banas, N. S., Bresnan, E., Burford, M. A., Davidson, K., Gobler, C. J., Karlson, B., Lim, P. T., MacKenzie, L., Montresor, M., Trainer, V. L., Usup, G., & Yin, K. (2017). GlobalHAB: A New Program to Promote International Research, Observations, and Modeling of Harmful Algal Blooms in Aquatic Systems. Oceanography, 30(1), 70. https://doi.org/10.5670/oceanog.2017.111

- Çelekli̇, A. (2024). The first eco-assessment of phytoplankton assemblages associated with environmental conditions of different lakes in the Western Black Sea basin of Türkiye. Research Square (Research Square). https://doi.org/10.21203/rs.3.rs-5078176/v1

- Devlin, A. T., Pan, J., Shah, M. M., Nuhu, A., Cummins, K. W., Zarges, C. V., Fierro, P., Olmos, V., Dirisu, A.-R., Olomukoro, J. O., Vieira, N., Soares, A., Pinheiro, C., Bingham, M. F., Kinnell, J., Bo, W., Zhang, J., Zhao, Y., Barbosa, H. A., … Agodzo, K. (2020). Inland Waters - Dynamics and Ecology. In IntechOpen eBooks. IntechOpen. https://doi.org/10.5772/intechopen.87463

- Everard, M., Sharma, O. P., Vishwakarma, V. K., Khandal, D., Sahu, Y., Bhatnagar, R., Singh, J. K., Kumar, R., Nawab, A., Kumar, A., Kumar, V., Kashyap, A., Pandey, D. N., & Pinder, A. C. (2017). Assessing the feasibility of integrating ecosystem-based with engineered water resource governance and management for water security in semi-arid landscapes: A case study in the Banas catchment, Rajasthan, India. The Science of The Total Environment, 612, 1249. https://doi.org/10.1016/j.scitotenv.2017.08.308

- Foulon, É., Rousseau, A. N., Benoy, G., & North, R. L. (2019). A global scan of how the issue of nutrient loading and harmful algal blooms is being addressed by governments, non-governmental organizations, and volunteers. Water Quality Research Journal, 55(1), 1. https://doi.org/10.2166/wqrj.2019.013

- Gobler, C. J., Drinkwater, R. W., Anthony, A. W., Goleski, J. A., Famularo-Pecora, A. M. E., Wallace, M. B., Straquadine, N. R. W., & Hem, R. (2024). Sewage-and fertilizer-derived nutrients alter the intensity, diversity, and toxicity of harmful cyanobacterial blooms in eutrophic lakes. Frontiers in Microbiology, 15, 1464686. https://doi.org/10.3389/fmicb.2024.1464686

- Haq, Z., Irshad, S., Khan, A. A., Ahmad, S. F., & Bashir, S. M. (2024). Advances in Managing Nitrogen and Phosphorus Emissions in Ruminants: A Holistic Approach. In Veterinary medicine and science. IntechOpen. https://doi.org/10.5772/intechopen.1006578

- Halpati, R. P., Bhusare, S., Sawant, K. S., Belsare, S., Devi, L. S., & Chandegara, A. K. (2026). Co-feeding using live and inert diets in aquaculture larval rearing: Nutritional significance, performance outcomes and future perspectives. Uttar Pradesh Journal of Zoology, 47(5), 42–52. https://doi.org/10.56557/upjoz/2026/v47i55534

- Haxthausen, K. R. A. (2021). Sediment microbial fuel cells for lake restoration. Research Portal Denmark, 76. https://local.forskningsportal.dk/local/dki-cgi/ws/cris-link?src=dtu&id=dtu-d66d3370-256f-4885-a09e-336a924f4bed&ti=Sediment%20microbial%20fuel%20cells%20for%20lake%20restoration

- Jain, S., Srivastava, A., Vishwakarma, D. K., Rajput, J., Rane, N. L., Salem, A. A., & Elbeltagi, A. (2024). Protecting ancient water harvesting technologies in India: strategies for climate adaptation and sustainable development with global lessons. Frontiers in Water, 6. https://doi.org/10.3389/frwa.2024.1441365

- Jansen, J., Simpson, G. L., Weyhenmeyer, G. A., Härkönen, L. H., Paterson, A. M., Giorgio, P. A. del, & Prairie, Y. T. (2024). Climate-driven deoxygenation of northern lakes. Nature Climate Change, 14(8), 832. https://doi.org/10.1038/s41558-024-02058-3

- Ji, D., Han, Y., Long, L., Xin, X., Xu, H., Qiu, S., Meng, J., Zhao, X., Huang, Y., & Liu, D. (2024). Hypoxia and its feedback response to algal blooms and CH4 emissions in subtropical reservoirs. Frontiers in Ecology and Evolution, 11. https://doi.org/10.3389/fevo.2023.1297047

- Karale, T., Savaliya, B., Raval, A., Debnath, S., & Bhusare, S. (2025). Aquaculture technology adoption in Tripura, India: Determinants, patterns and challenges. Journal of Experimental Agriculture International, 47(10), 361–370. https://doi.org/10.9734/jeai/2025/v47i103819

- Jilbert, T., Couture, R., Huser, B., & Salonen, K. (2020). Preface: Restoration of eutrophic lakes: current practices and future challenges. Hydrobiologia, 847(21), 4343. https://doi.org/10.1007/s10750-020-04457-x

- Devi, L. S., Waikhom, D., Karale, T., Singh, A. B., Rasal, K., & Singh, A. L. (2025). Innovations in modern aquaculture: A review on advanced reproductive technologies. Uttar Pradesh Journal of Zoology, 46(23), 41–67. https://doi.org/10.56557/upjoz/2025/v46i235382

- Khorasani, H., & Zhu, Z. (2021). Phosphorus retention in lakes: A critical reassessment of hypotheses and static models. Journal of Hydrology, 603, 126886. https://doi.org/10.1016/j.jhydrol.2021.126886

- Aniket, F., Karale, T., Kumari, I., Avinash, C., & Devi, L. S. (2025). Rupchand (Piaractus brachypomus) farming in India: Prospects, challenges and sustainability implications. International Journal of Environment and Climate Change, 15(10), 403–411. https://doi.org/10.9734/ijecc/2025/v15i10507

- Kumar, P., & Mahajan, A. K. (2020). Trophic status and its regulating factor determination at the Rewalsar Lake, northwest Himalaya (HP), India, based on selected parameters and multivariate statistical analysis. SN Applied Sciences, 2(7). https://doi.org/10.1007/s42452-020-3082-8

- Raval, A. D., Das, B. K., Savaliya, B., Tandel, M. A., Ad, V., Koley, S., Karale, T., & Singa, S. (2025). Histological alterations and recovery response in Labeo rohita exposed to extreme acidic and alkaline pH conditions. Uttar Pradesh Journal of Zoology, 46(24), 85–94. https://doi.org/10.56557/upjoz/2025/v46i245409

- Lewis, A. S. L., Lau, M. P., Jane, S. F., Rose, K. C., Be’eri-Shlevin, Y., Burnet, S. H., Clayer, F., Feuchtmayr, H., Grossart, H., Howard, D. W., Mariash, H., Martín, J. D., North, R. L., Oleksy, I. A., Pilla, R. M., Smagula, A. P., Sommaruga, R., Steiner, S. E., Verburg, P., … Carey, C. C. (2023). Anoxia begets anoxia: A positive feedback to the deoxygenation of temperate lakes. Global Change Biology, 30(1). https://doi.org/10.1111/gcb.17046

- Meerhoff, M., Audet, J., Davidson, T. A., Meester, L. D., Hilt, S., Kosten, S., Liu, Z., Mazzeo, N., Paerl, H. W., Scheffer, M., & Jeppesen, E. (2022). Feedback between climate change and eutrophication: revisiting the allied attack concept and how to strike back. Carolina Digital Repository (University of North Carolina at Chapel Hill). https://doi.org/10.17615/7ve5-w016

- Savaliya, B. D., Renuka, T. K., Bhusare, S., Halpati, R. P., Chandegara, A. K., & Chauhan, J. K. (2026). Sea change: Reshaping global fisheries—A review. Indian Research Journal of Extension Education, 26(1). https://doi.org/10.54986/irjee/2026/jan_mar/82-91

- Nair, N. V., & Nayak, P. K. (2023). Uncovering water quality and evaluating vulnerabilities of small-scale fisheries in Chilika Lagoon, India. Frontiers in Marine Science, 10. https://doi.org/10.3389/fmars.2023.1087296

- Nõges, T., Janatian, N., Laugaste, R., & Nõges, P. (2020). Post-soviet changes in nitrogen and phosphorus stoichiometry in two large non-stratified lakes and the impact on phytoplankton. Global Ecology and Conservation, 24. https://doi.org/10.1016/j.gecco.2020.e01369

- Paerl, H. W. (2018). Mitigating toxic planktonic cyanobacterial blooms in aquatic ecosystems facing increasing anthropogenic and climatic pressures. Carolina Digital Repository (University of North Carolina at Chapel Hill). https://doi.org/10.17615/yn55-b438

- Karale, T., Dalavi, P., Savaliya, B., Devi, L. S., & Bhusare, S. (2025). Impact of various pre-freezing treatments on drip loss of tilapia (Oreochromis niloticus) fillets during long-term storage at −18°C. European Journal of Nutrition and Food Safety, 17(11), 131–138. https://doi.org/10.9734/ejnfs/2025/v17i111899

- Paerl, H. W., Havens, K. E., Hall, N. S., Otten, T. G., Zhu, M., Xu, H., Zhu, G., & Qin, B. (2019). Mitigating a global expansion of toxic cyanobacterial blooms: confounding effects and challenges posed by climate change. Marine and Freshwater Research, 71(5), 579. https://doi.org/10.1071/mf18392

- Savaliya, B. D., Solomon, J., Karale, T., Kumar, G., Paul, T., Kumar, S., Bhusare, S., & Priyadharshini, T. (2026). Biodegradability assessment of aquaculture effluents: A comprehensive analysis of BOD and COD using multivariate statistical techniques. International Journal of Bio-Resource & Stress Management, 17(2). https://doi.org/10.23910/1.2026.6871

- Paerl, H. W., Havens, K. E., Hall, N. S., Otten, T. G., Zhu, M., Xu, H., Zhu, G., & Qin, B. (2020). Mitigating a global expansion of toxic cyanobacterial blooms: Confounding effects and challenges posed by climate change. Carolina Digital Repository (University of North Carolina at Chapel Hill). https://doi.org/10.17615/4nkd-zk93

- Savaliya, B. D., Karale, T., Bhusare, S., Gautam, R., Ad, V., & Raval, A. D. (2025). Comparative analysis of carbon emissions from trawl fisheries in India within a global framework. International Journal of Environment and Climate Change, 15(12), 746–755. https://doi.org/10.9734/ijecc/2025/v15i125193

- Ronoh, E., Schlemm, A., Mkama, E. B., Nyolei, D., Nyongesa, J. M., Nyadawa, M., & Griensven, A. van. (2025). Application of Nature-Based Solutions (NbS) for Catchment Water Quality Management in Yala River Basin, Western Kenya. https://doi.org/10.5194/egusphere-egu25-18259

- Sampat, A. M., Hicks, A., Ruiz‐Mercado, G. J., & Zavala, V. M. (2020). Valuing economic impact reductions of nutrient pollution from livestock waste. Resources Conservation and Recycling, 164, 105199. https://doi.org/10.1016/j.resconrec.2020.105199

- Bhusare, S., Halpati, R. P., Sawant, K., Belsare, S., Palke, A. A., Raghuvaran, N., & Ram, A. R. (2026). Black soldier fly larvae meal as an alternative to fishmeal in aquaculture nutrition. European Journal of Nutrition & Food Safety, 18(3), 87–98. https://doi.org/10.9734/ejnfs/2026/v18i31977

- Shahbaz, M., Alam, A., Zafar, M. M., Sulaiman, M. A., Kumari, A., Sharma, G. K., & Yasmin, S. (2023). Comparison of avian diversity between managed and unmanaged wetlands in Patna, Bihar, India. Ornis Hungarica, 31(2), 13. https://doi.org/10.2478/orhu-2023-0016

- Shahnazaryan, G., Schultze, M., Rinke, K., & Gabrielyan, B. (2023). Lake Sevan. Past, present, and future state of a unique alpine lake. Journal of Limnology, 81. https://doi.org/10.4081/jlimnol.2022.2168

- Satkar, S. G., Kumar, A., Bhusare, S., Sahu, A., & Gautam, R. K. (2024). Impact of pollution and toxic stress on fish health: Mechanisms, consequences, and mitigation strategies. Uttar Pradesh Journal of Zoology, 45(6), 29–45. https://doi.org/10.56557/UPJOZ/2024/v45i63949

- Simon, D. F., Munoz, G., Dinh, Q. T., Duy, S. V., Kavanagh, K., Smith, R. J., Husk, B., & Sauvé, S. (2023). Adopt a Lake: Successfully Tracking Harmful Cyanobacterial Blooms in Canadian Surface Waters Through Citizen Science. Citizen Science Theory and Practice, 8(1), 66. https://doi.org/10.5334/cstp.655

- Singh, S. K. (2025). Rainwater Harvesting Systems in Urban and Rural India: Performance and Policy Review. International Journal for Research in Applied Science and Engineering Technology, 13(7), 1165. https://doi.org/10.22214/ijraset.2025.73128

- Sitote, Y. M., & Gebremedhine, M. G. (2024). Comprehensive Review of Eutrophication in Freshwater Ecosystems: Causes, Effects, Assessment, and Management Strategies. Preprints.Org. https://doi.org/10.20944/preprints202409.0704.v1

- Swann, M. M., Cortés, A., Forrest, A. L., Framsted, N. T., Sadro, S., Schladow, G., & Palma‐Dow, A. D. (2024). Internal phosphorus loading alters nutrient limitation and contributes to cyanobacterial blooms in a polymictic lake. Aquatic Sciences, 86(2). https://doi.org/10.1007/s00027-024-01045-2

- Wang, X., Shi, K., Zhang, Y., Qin, B., Zhang, Y., Wang, W., Woolway, R. I., Piao, S., & Jeppesen, E. (2023). Lake warming and increased heatwaves revealed by a novel data-driven modeling approach. Research Square (Research Square). https://doi.org/10.21203/rs.3.rs-2442142/v1