Short Research Article

Evaluation of the performance of a central pivot in an irrigation system in the Agricultural Unit of Borotou-Koro

ABSTRACT

Importance of the work : The performance of an irrigation system is determined by its ability to ensure uniform water distribution in the areas covered. It is assessed using the system's uniformity coefficient.

Objectives : The aim of this study is to assess the uniformity coefficient of a pivoting boom in two sectors of the Borotou-Koro sugar complex.

Place and Duration of Study: The study was carried out at the Borotou-koro integrated agricultural unit, irrigation section, from March 2024 to June 2024

Materials and Methods: To carry out this study, we used the ‘Heermann and Hein’ method, which is based on the American standard ASAE/ANSI S436.1.  This method consists of estimating the volume of water collected in various collectors located at equal distances from the pivot, weighted by the difference in surface area attributed to each collector using a specific equation.

Results According to the data analysis, 95% of pivots have an excellent or good uniformity coefficient (UC), while less than 2% have a poor UC. The study also reveals a lack of systematic inspection of the systems to detect nozzle malfunctions in a timely manner.

Main finding It should be noted that the UC is a key indicator for gauging the performance of the pivot system in terms of consistent water supply to crops such as sugar cane, whose yield is heavily dependent on this factor of production.

Mot clés Borotou-koro locality, Irrigation, Sugar complex,  Uniformity coefficient,

1. INTRODUCTION

Water is one of the essential factors in agricultural production. A shortage of water is a limiting factor in this production. Consequently, irregular rainfall cannot meet the water needs of the most water-intensive crops. This is particularly the case for sugar cane, which cannot tolerate irregular rainfall patterns linked to climatic variability [1] To cope with climatic hazards and ensure good production, [2] recommends irrigation as a viable alternative. Although this practice involves several methods, it has the same objective. The aim is to meet the water requirements of cultivated plants [3]. According to [4] estimates of plant water requirements are in the order of 1,700 to 2,300 m³/ha/year. However, the performance of irrigation systems, estimated by the uniformity coefficient, should be in the order of 90 to 95% [5] Sprinkler irrigation, which holds great promise for large-scale crops, has  gradually been adopted by farmers [6].. This form of irrigation exists in a wide range of more or less sophisticated forms, from manually movable sprinkler booms to  pivoting booms. [7]  describes this device as a system equipped with a water column and a pivoting ramp that distributes water in a circular pattern around a central axis.  According to [8], the operating principle of sprinkler irrigation previously required the development of a platform to anchor the pivot arm. Nowadays, one of the modern irrigation systems used in agriculture is the centre pivot, which makes better use of water resources [9]. It adapts to all types of terrain and ensures production during periods of water shortage [10]. However, this system can present a potential risk of clogging or obstruction of the nozzles. Hence the need to evaluate its performance periodically in order to make the necessary adjustments. This study, which is part of this initiative, aims to evaluate the performance of the pivot irrigation system used by the Sucrivoire company in the Borotou-Koro agricultural unit in Ivory Coast.

2 MATERIAL AND METHODS

2-1. Presentation of the study area

The Borotou-Koro sugar complex, located at geographical coordinates 8°28'37‘W and 7°11'20’N, is situated in north-western Ivory Coast. It takes its name from the locality where it is located (Figure 1: red box). The locality is crossed by tributaries of the Sassandra River, including the Boa and Gbagbé rivers, which serve as a source of water for irrigating the sugar complex's plots. The soils of the sugar complex are ferralitic.

Figure 1 : location of the study site (red box) in the Woroba district (in green) (Auther : Lahouri, 2024)

2-2. Technical data

The Ivory Coast's sugar requirements are estimated at 200,000 tonnes, with annual production estimated at between 150,000 and 180,000 tonnes per year ([11]. To make up for this shortfall and ensure sustainable sugar production, Ivorian sugar companies have opted to expand production areas from 25,400 ha to 61,400 ha [11]. However, the sugar cane industry is facing the thorny issue of climate change, which requires improvements in production factors such as water management. With an average temperature of 27°C, the production area enjoys a monomodal climate characterised by a dry season (November to March) and a rainy season (April to October). Monthly rainfall averages range from 102 to 258 mm, with an annual average of around 1,334 mm [11], while sugarcane requires between 1,500 and 2,500 mm of water for optimal yield. To bridge the gap, a pivot irrigation system has been installed and must undergo periodic evaluation tests to maintain its performance. Although the topography is moderate, the clay-sandy soils are vulnerable to erosion and can therefore be a source of soil pollution in the pumped water, with the immediate consequence of increased turbidity. These soil and climate conditions led to the choice of a pivot system with an average length of 500 m and an average of 8 spans. The average flow rate is 250 m3/h, covering an area of 70 ha in rotation for an average of 20 hours. The average pressure is set at 2,50 bar to prevent excessive soil erosion, which causes nozzle clogging that affects the system's performance, resulting in lower sugar cane yields.

2-3. Study data

The volume of water collected in the collectors at different distances from the pivot constitute the data used to calculate the uniformity test.

2-4. Technical equipment

It onsists of data collection equipment that can be used to calculate the uniformity coefficient (Figure 2) :

 Tape measure, to determine the positioning of the ‘water collectors’ from the central pivot (image A);

 Pegs, to mark the positioning of the ‘rain collectors’ (image B);

 ‘Water collector with funnel’ (in blue), to collect irrigation water from the pivot ramps (image C);

 Graduated cylinder, to measure the height of the water collected in the ‘rain collector’ (image D).

 Pegs, to mark the positioning of the ‘rain collectors’ (image B);

 ‘Water collector with funnel’ (in blue), to collect irrigation water from the pivot ramps (image C);

 Graduated cylinder, to measure the height of the water collected in the ‘rain collector’ (image D).

Decameter

Stakes

Collector

Graduated cynlinder

Decameter

Stakes

Collector

Graduated cynlinder

Figure 2 : Some of the technical equipment used to collect field data (A-décameter ; B Stakes ; C- rain collector; D- Graduated cylinder) (Auther :  Lahouri, 2024)

2-5. Study data collection

The experimental device is a fixed unit, known as a central pivot, which supports a rotating structure composed of several spans equipped with pipes and nozzles, allowing the pivot to cover the plot by forming concentric arcs. Water is pumped from a river and transported along the pipes to the nozzles, which spray the water in the form of droplets. The layout of the water collectors in relation to the pivot and how they work are shown in Figures 3 and 4. Figure 4 shows a gauge for measuring the volume of water collected.

In practice, the “water collectors” were placed at regular intervals along the line of stakes. Their number varies depending on the range of the pivot. The first “water collector” is placed 15 meters from the central unit. The distance between the two “water collectors” on lines 1 and 2 is 90 cm. The interval between two consecutive collectors on the same line is 10 m. The assembly forms a cone around the central pivot. The last stakes are separated from neighboring lines by a distance of 10 to 50 m. At the end of this operation, the irrigation pivot is run at maximum capacity (100%) for a specified period. The flow rate is low near the pivot and maximum at the end. At the end, the amount of water collected by the “rain collector” is measured in a graduated cylinder. The calculation of efficiency takes into account the height of the water, the distance between the water collectors and the center of the pivot, and the average height of the water collected. The area covered by each “water collector” is proportional to the radius of the spacing between the collectors.

.

Figure 3. Schematic diagram of the experimental setup of a center pivot irrigation system (black dots represent water collectors).”

Figure 4. Stake and rain collector                                Figure 5. Water level reading

(auther : E. Lahouri, 2024)

2-6. Uniformity coefficient estimate

The uniformity of irrigation from the pivoting ramp is obtained using a water distribution test. In this work, [13] attributes the best yields to the uniformity of water distribution in the irrigation systems used. The uniformity coefficient (UC) of the system is calculated from the amount of water collected in each collector. The method used to calculate this coefficient is that of ‘Heermann and Hein’ based on the American standard [141 and defined by :

CU =100×1-i=1i=nSi.Di-Dpi=1i=nSi.Di       Equation 1; Where,

CU= Uniformity Coefficient, n = is the water collector in question, Di = water depth or volume of water collected in the collectors, D ̅p = average volume of water collected    , Si =Interception area of the collector

3. RESULTS AND DISCUSSION

The results obtained from the analysis of 59 pivots, including 42 in sectors B and 17 in sector C, show two (02) highly significant classes with p<0,00001, indicating low dispersion and therefore homogeneous data.  The UC (%) is therefore stable and well controlled in this class. Class 1, which covers 52 observations, has a uniformity coefficient value of 91, 01% ± 1,637, while class 2, with 7 observations, shows lower and more dispersed values, with a mean of 81,72% ± 1,899 (Table 1). This difference could be explained by nozzle malfunctions (leaks or blockages).

Table 1: Descriptive analysis

| Variable | SC Inter |  | Dl | SC Intra | DI | F | Signif p |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Classe UC (%) | 312,033 |  | 1 | 110,063 | 40 | 113,403 | ˂0,00001 |
| UC (%) Class 1 |  | Descriptive statistics for 52 observations |  |  |  |  |  |
|  |  | Average |  | Standard deviation |  | Variance |  |
|  |  | 91,01% |  | 1,637% |  | 2,682% |  |
| UC (%) Class 2 |  | Descriptive statistics for 07 observations |  |  |  |  |  |
|  |  | Average |  | Standard deviation |  | Variance |  |
|  |  | 81,72% |  | 1,899% |  | 3,609% |  |

SC Inter: sum of inter-class squares; SC Intra: sum of intra-class squares; Dl: degrees of freedom; F: Fisher's statistic; p: probability associated with the test.; UC (%) : uniformity coefficient class.

In addition, the analysis of the results of uniformity tests for sectors B and C of the complex splot (Table 2 and 3) shows that :

 64 ,3% of pivots in sector B, and 58,8% of those in sector C had an excellent uniformity coefficient (UC) of over 90%, while

 26,2% of the pivots in sector B, and 3,.3% of those in sector C gave a good uniformity coefficient (UC) (value between 85 and 90%)

.  Table 2: Efficiency rate of the uniformity coefficients of the various pivots in sector B

| Sector B | Excellent | good | fair | poor | Total |
| --- | --- | --- | --- | --- | --- |
| Number of pivot | 27 | 11 | 03 | 01 | 42 |
| UC | +90% | 90% | +80% | -80% |  |
| rate (%) | 64,3 | 26,2 | 7,1 | 2,4 | 100 |

Table 3 : Efficiency rate of the uniformity coefficients of the various pivots in sector C

| Sector C | Excellent | good | fair | poor | Total |
| --- | --- | --- | --- | --- | --- |
| Number of pivot | 10 | 06 | 01 | 00 | 17 |
| UC | +90% | 90% | +80% |  |  |
| rate (%) | 58,8 | 35,3 | 5,9 | 00 | 100 |

These results for these two sectors show good to excellent uniformity coefficients, reflecting the good performance of the irrigation system deployed in these sectors of the complex. These observations are identical to those in the work of [15] cited by [16]  In his study, the author specifies that all uniformity coefficient values above the threshold of 80% are good to excellent in terms of performance or efficiency. However, this does not exclude the need for periodic monitoring to detect possible malfunctions and ensure system maintenance. In this study, it appears that

- 7,1% of pivots in sector B, and 5 ;9% of pivots in sector C have a uniformity coefficient (UC) between 80 and 85%, which means that more than 90% of pivots in both sectors have a uniformity coefficient of more than 90%; evidence that irrigation is acceptable, but control needs to be strengthened.

- 2,4% of pivots in sector B alone have a uniformity coefficient (UC) of less than 80%, which could be the cause of poor water distribution uniformity.These observations are consistent with those of [17] , who classify networks in good condition as those with a UC above the 80% threshold, networks requiring cleaning as those with a uniformity coefficient between 70% and 90%, and clogged networks as those with a UC  below 70%. Overall, 97,6% (i.e. 40 pivots) of the pivots in sector B had a UC above 90% and only 2,4% (i.e. 2 pivots) had a UC below 80%. In sector C, all 17 pivots had an excellent to good UC (i.e. a rate of 100%). These results show that the system deployed ensures good water coverage of the plots. According to the work of [18] checking the condition of the nozzles can help detect clogging, leaks, blockages or wear and tear, and maintenance can be carried out to ensure the efficiency of the irrigation system. This system diagnostic operation must therefore be carried out at least once a year to ensure that mechanised water sprinkler systems are functioning properly.  In addition, a soil characterisation study (permeability) should be included in irrigation efficiency studies, as this factor influences the availability of water for plants

4. CONCLUSION

This study shows that the irrigation system deployed in sectors B and C is generally effective in terms of uniformity coefficients, which are good to excellent (above 90%) according to the data collected. The results reveal some weaknesses attributable either to leaks or to nozzle clogging. Maintenance operations are aspects that need to be integrated into the process in order to maintain the performance of the irrigation system. In addition, it is important to include factors such as wind and soil permeability, which affect irrigation, in future studies evaluating irrigation performance.

5. BIBLIOGRAPHIC REFERENCES

[1]  MARE-DGGRRE (Ministry of Agriculture and Water Resources, General Directorate for  Rural Engineering and Water Resources), (2017). Preparation report for the investment project in the water sector, 25p.

https://planbleu.org/sites/default/files/upload/files/Rapport_national_Eau_TN.pdf

[2]  M’Sadak Y., Douh B., Hamza M. E., (2012). Functional diagnosis of pivot sprinkler systems used on a large irrigated farm in central Tunisia. Larhyss Journal (11): 97-107.

https://www.google.com/url?sa=t&source=web&rct=j&opi=89978449&url=https://www.asjp.cer232ist.dz/en/downArticle/11/7/1/40094&ved=2ahUKEwiD6tfT5vyHAxWgRaQEHbwaJkYQFnoEC234 BMQAQ&usg=AOvVaw3V0DHOf4c7AvmrxJIvoTmZ

[3]  Kambou D., Xanthoulis D., Ouattara K., Degre A. (2014). Concepts of water efficiency and productivity (bibliographical synthesis), BASE, Université de Liège, V 18 No 1, p.

108-120.https://popups.uliege.be/1780-4507/index.php?id=16825&file=1&pid=10862 [4] Dunglas J. (2014). Irrigation techniques. Water Group Working Notes No. 6, French Academy of Agriculture, 6 p. https://www.asjp.crist.dz.

[5]  Rieul L. (1997). Future irrigation techniques and their cost. In : Dupuy B., ed. Economic 239 aspects of water management in the Mediterranean basin. Bari, Italie, CIHEAM, p. 233-240.

[6]  Abla K., Ali H., Abdelilah T., Marcel K.. (2020) L’expérience de l’irrigation par aspersion dans les secteurs collectifs de la Grande Hydraulique du Maroc. Revue Marocaine des Sciences Agronomiques et Vétérinaires, 2020, 8 (4), pp.399-410. ⟨hal-04561263⟩ https://hal.science/hal-04561263/document

[7]   Phocaides A. (2008). Manual of pressure irrigation techniques, Second edition. FAO, 308  p. // http://www.fao.org/nr/water/ies/

108-120.https://popups.uliege.be/1780-4507/index.php?id=16825&file=1&pid=10862

[8]  Molle B., Baudequin D., Haidar S. (1998). Selection of pipes and management of centre-pivot irrigation – Taking soil infiltration capacity into account. Ingénieries Eau Agriculture & Territoires (EAT), IRSTEA, pp. 57–69

https://www.researchgate.net/publication/41662476_Choix_du_busage_et_conduite_de_l'irrigation_par_pivots_-_Prise_en_compte_de_la_capacite_d'infiltration_du_sol

[9]Aït Houssa A., Bouslama A., Baraka M., El Midaoui, M. and Benbella M. (2009). The use of pivot centers for irrigation: Experience of Agricultural Estates of Morocco, Technology Transfer in Agriculture. Monthly Liaison and Information Bulletin of the PNTTA, N°177. https://www.agrimaroc.net/bulletins/btta_177.pdf

[10]  Hutchinson C.F.and Hermann S.M. (2006). Plant Science. Quarterly Information Bulletin on Exact and Natural Sciences, Vol. 4, N°4. https://typeset.io/pdf/diagnostic-de-fonctionnement-des-systemes-d-aspersion-par wdpgo3zjiz.pdf

[11]  Konate Y., Zilé K.Al., Eba C. E. and Wawogninlin C. B., (2025). Influence of Agroclimatic Parameters on Sugar Cane Production: Case Study of the Borotou Koro Integrated Agricultural Unit, North-West Côte d'Ivoire, pp. 6-20

. https://doi.org/10.36948/ijfmr.2025.v07i02.38846

[12]ASAE/ANSI S4361 DEC 01( 2003). Test  procedure of determining the uniformity of water distribution of center pivot and lateral move irrigation machines equiped with spray or sprinkler nozzles. https://www.canr.msu.edu/uploads/235/67987/ASAE_S436.1.pdf

[13]  Burt C., (2000). Proceedings of the Water Resources Div., ASCE, Annual Conference. 214 Memphis, Tenn. Selection of Irrigation Methods for Agriculture : Drip/Micro Irrigation 215 TRC Paper 98-004

http://www.itrc.org/papers/selecofirrmethod/selectionofirrmethod.pdf

[14]  Van der Gulik P.S., Richard Mostert  and van den Berg H.R. (1989).The working equations of a vibrating wire viscometer, https://doi.org/10.1016/0378-4371(89)90027-7

[15] Bergeron D. et J. Noreau. 2006. Régie de l'eau - Irrigation goutte à goutte. Producteurs Plus - Zoom Légumes, 14-18.

file:///C:/Users/HP/Downloads/27505%20(1).pdf

[16]  Keller J. Et Karmeli D. (1974). Trickle Irrigation Design Parameters. Transactions of American Society of Agricultural Engineers, 17, 678-684.

http://dx.doi.org/10.13031/2013.36936

[17] Douh B., Chehaibi S. and Zouaghi M., (2012). Diagnostic analysis of the maintenance of  mechanized sprinkler irrigation systems case of a frontal ramp in the region of Bousselem,  Tunisia. Larhyss Journal (11): 47-61. https://typeset.io/pdf/analyse-diagnostique-du-maintien-en-etat-de-fonctionnement 57017cftt7.pdf