# “Evaluation of Physico-Chemical Characteristics of Water in Pariyaram Panchayath, Thrissur District, Following the 2018 Kerala Flood”

## Abstract

Floods are among the most destructive natural disasters, significantly affecting water quality and ecosystem stability. The present study evaluated the impact of the 2018 Kerala flood on the physico-chemical characteristics of water in flood-affected and unaffected areas of Pariyaram Panchayat, Thrissur district. Water samples were collected from affected wards and a control (unaffected) ward and analyzed for key parameters including pH, electrical conductivity (EC), total dissolved solids (TDS), turbidity, alkalinity, hardness, chloride, phosphate, and nitrate using standard analytical methods. The results revealed a significant (p < 0.05) decrease in pH, turbidity, and alkalinity, along with a significant increase in chloride levels in flood-affected areas compared to the control. However, EC, TDS, hardness, phosphate, and nitrate did not show significant variation between the two groups. Despite these changes, all measured parameters remained within permissible limits of international drinking water standards. The observed variations indicate the influence of flood-induced processes such as organic matter decomposition, sedimentation, and contamination from surface runoff. Overall, the findings suggest that while floods can alter water quality, the long-term impacts may be moderate due to natural recovery mechanisms. Continuous monitoring and appropriate management strategies are essential to ensure safe water for livestock and human consumption in flood-prone regions.

## Keywords: Flood impact; Water quality; Kerala flood 2018; Physio-chemical parameters; Drinking water standards; Environmental monitoring; Livestock health; Runoff contamination

Introduction

India is vulnerable to multiple forms of natural disasters. As per Indian Metrological Department (IMD) 2018, India is second only to Bangladesh with regard to total magnitude of such events the word over.  Two rainy seasons, first being the Southwest monsoon that starts by the end of the May or early June also called as Edavapathi and Northeast monsoon starts in mid-October also called as Thulam. State lying between high Western Ghats and the Arabian sea, which is located at the southernmost tip of Indian peninsula, has 50 major dams and 44 major rivers runs towards west and has innumerable streams and backwaters owing heavy rainfall during the monsoon (Ramasamy et al., 2019). As per IMD-2018 report data, state which received 2346.6 mm of rainfall which was expected as 1649.5 mm which was 42 per cent more than normal rainfall (1197 mm) and nearly 500 landslides were occurred. In that short period of 30 days, 339 people were died, thousands of living area damaged, nearly one million people stayed in relief camps and many roads, bridges were damaged. Uninterrupted rains lashed most areas of the State from 8th to 18th of August 2018 which resulted in wide spread destruction in all the major sectors of the state. This indicates Southwest season can be cited as an example of global climate change impact with very heavy rainfall in a short span of time as indicated and predicted by the Fifth Assessment Report published by the Intergovernmental Panel for Climate Change (IPCC) in 2014. This flood affects the entire major sectors especially agriculture which provides 15 per cent of Gross Domestic Product of India and 70 per cent of people depend on agricultural activity (Central Statistical office report, 2018-19). Pariyaram is one among the Grama panchayat in Thrissur was severely affected by Kerala flood. This climate change had made severe impact to ecosystem including farming system and also water and soil quality. So this change should be rectified to assess the condition in flood affected area. Based on this situation, this study had been taken to assess the impact of Kerala flood 2018 on livestock farming system of Pariyaram Panchayat in Thrissur district of Kerala.

Materials and Method

Location of study

Pariyaram Panchayat was located at 10.3200ºN, 76.3705ºE in Thrissur district near Chalakkudy taluk with 16.8 km2 area and population covering of 31195 (2011 census). Due to its topographic peculiarity the Panchayat was affected by landslides as well as floods during 2018. Out of 15 wards, Peelarmuzhi was not affected by flood and landslides during 2018, remaining 14 wards were affected at varying severity. So the unaffected ward was considered as control for the study. The Panchayat has hilly, plain and riverine zones.

Procedure for water sampling

The samples were collected in 250ml sterilized plastic containers, rinsed with distilled water and 75% alcohol and finally washed with de-ionized water to ensure that the containers are physically and chemically clean. At each sampling point, the containers rinsed three times with the water sample to be analyzed and sealed properly. Samples were analyzed in laboratory approximately 24 hours after collection of sample. The results were compared with control value and WHO standards for drinking water (Acharya, 2018). The parameters analyzed was pH, Electrical conductivity, Total Dissolved Solids, Turbidity, Hardness, Alkalinity, Chloride, Phosphate and Nitrate

ANALYSIS OF WATER SAMPLE

pH

pH was determined using hand held pH meter (Model-Eco Testr PH2 meter). pH was noted at the time of sample collection by inserting the probe for one to two minutes. (American Society for Testing and Materials- ASTM, 2003)

ELECTRICAL CONDUCTIVITY

Electrical conductivity was determined using EC meter (Model-COM-80 HM DIGITAL meter). Meter was stabilized and reading was recorded. EC was expressed as µS/cm. (ASTM, 2003)

TURBIDITY

Turbidity was determined by Photometric method using HACH DR/2010 spectrometer at a wavelength of 860nm and programme number 750. Reading was in Neplometric Turbidity Unit (NTU) (Emeka and Osondu, 2016)

Procedure: Mix the sample to thoroughly disperse the solids. Wait until air bubbles disappeared then pour the sample into the turbidity meter tube. Read the turbidity directly from the instrument scale.

TOTAL DWASSOLVED SOLIDS (TDS)

Total dissolved solids were measured by using TDS meter (COM-80 Digital Meter). (ASTM, 2003)

Procedure: TDS was noted at the time of collection by inserting the probe for one to two minutes.

TOTAL ALKALINITY

Total alkalinity was determined by titrating a known volume of water with standard sulphuric acid using phenolphthalein and methyl orange as indicators. (ASTM, 2003)

CHLORIDE

Chloride was analyzed by titrating a known volume of water with standard silver nitrate solution using potassium chromate as indicator (Mohr’s Method) (ASTM, 2003)

HADRNESS

Fifty ml of water sample was buffered at pH 8 -10 (NH4Cl and NH4OH) and titrated against standard EDTA using Eri-chrome Black T as indicator (APHA, 1989)

PHOSPHATE

Phosphate was estimated using stannous chloride method (Joshi et al., 2009)

Preparing reagents

All chemicals used were of SQ grade purchased from Qualigen, Thermo Fisher Scientific, Mumbai. Reagents and working solutions were prepared using double glass distilled water. Stock phosphate solution of 500 mg PO4/lit was prepared by using potassium dihydrogen phosphate. Working standard solution of 50 mg PO4/lit was prepared from above stock which was further used for preparation of standard series of phosphate.

Procedure

All types of phosphates are finally converted to orthophosphate which was inorganic soluble form of phosphate. Phosphorous occurred as orthophosphate can be measured calorimetrically. In present study the microcontroller based system was verified (calibrated and standardized) with stannous chloride colorimetric method of inorganic phosphate estimation. Standard series of phosphate was prepared and absorbance was measured using Equiptronics EQ650 colorimeter. In acidic conditions phosphorous occurring as orthophosphate reacts with ammonium molybdate to form molybdo phosphoric acid. It was reduced by stannous chloride to a blue color complex. The intensity of blue color was measured which was directly proportional to concentration of phosphate in the water sample (Maiti, 2016). Samples were run in parallel with the microcontroller system for comparison of performance. The experiment was run in triplicate to improve accuracy level of detection and minimizing errors. Recording was done in mg/L

3.4.9 NITRATE

Nitrate was analysed by spectrophotometry (Sun et al., 2016)

Procedure

In a 100 ml beaker, 10 ml of an aliquot of the sample was taken and evaporated to dryness. To the dry residue, 2 ml of Phenol Disulphonic Acid (PDA) reagent was added. Then 10 ml of concentrated NH4OH was added carefully in a fume hood. Finally, the content was made up to 100 ml by using distilled water. The absorbance of the sample was read by using a UV spectrometer at 410 nm. In the similar way the calibration curve was prepared by using standard nitrate (10 ppm to 50 ppm) solution and blank solution (for the blank, distilled water was used in place of the sample). Corresponding value was recorded in mg/L. Maximum permissible limits in potable water was 45 mg/L.

Statistical analysis

Statistical analysis of the data was carried out using the analysis of variance and Wilcoxon signed rank test. Software used was SPSS V 24.0

PHYSICAL QUALITY OF WATER IN FLOOD AFFECTED AND UNAFFECTED AREAS

pH

Table 1, shows that pH of water from control area (6.433 ± 0.24) was significantly different from affected areas (5.982 ± 0.05).

Electrical Conductivity (EC)

Table 1, shows that EC of water from the control area (62.33 ± 2.29) was not significantly different from the affected areas (64.50 ± 2.14).

Total Dissolved Solids (TDS)

From the table 1, we can observe that TDS of water from control area (398.93 ± 14.65) was not significantly different from affected areas (401.73 ± 10.75)

Turbidity

From the table 1, we can observe that turbidity of water from control area (1.01 ± 0.21) was significantly different from affected areas (0.52 ± 0.05).

CHEMICAL QUALITY OF WATER IN FLOOD AFFECTED AND UNAFFECTED AREA

Phosphate

Table 2 shows that, there was no significant difference between the control area (0.16 ± 0.02) from the affected areas (0.30 ± 0.03) with regard to phosphate in water.

Nitrate

Table 2 shows that, there was no significant difference between the control area (6.41 ± 0.52) from the affected areas (6.68 ± 0.13) regarding nitrate in water.

Hardness

From the table 2 we can observe the hardness of water from control area (20.00 ± 1.15) was not significantly different from affected areas (22.24 ± 0.73).

Alkalinity

From the table 2, we can observe the alkalinity of water from control area (320.00 ± 25.82) was significantly different from affected areas (189.71 ± 8.2)

Chloride

From the table 2, we can observe the chloride of the water indicates that control area (141.40 ± 5.14) was significantly different from affected areas (191.59 ± 7.13).

Table 1: Physical quality of water in affected and unaffected area

| PARAMETERS | CONTROL |  | AFFECTED AREA |
| --- | --- | --- | --- |
| pH | 6.433 ± 0.24a | 5.982 ± 0.05b |  |
| Electrical Conductivity (µS/cm) | 62.33 ± 2.29a | 64.50 ± 2.14a |  |
| Turbidity (NTU) | 1.01 ± 0.21a | 0.52 ± 0.05b |  |
| Total Dissolved Solids (mg/L) | 398.93 ± 14.65a | 401.73 ± 10.75a |  |

For each parameter, means with different superscript (a-b in rows) differ

Significantly (p < 0.05)

Table 2: Chemical quality of water in affected and unaffected area

| PARAMETERS | CONTROL |  | AFFECTED AREA |
| --- | --- | --- | --- |
| Phosphate (mg/L) | 0.16 ± 0.02a | 0.30 ± 0.03a |  |
| Nitrate (mg/L) | 6.42 ± 0.52a | 6.68 ± 0.13a |  |
| Alkalinity (mg/L) | 320.00 ± 25.82a | 189.71 ± 8.2b |  |
| Hardness (mg/L) | 20.00 ± 1.15a | 22.24 ± 0.73a |  |
| Chloride (mg/L) | 141.40 ± 5.14a | 191.59 ± 7.13b |  |

For each parameter, means with different superscript (a-b in rows) differ

Significantly (p < 0.05)

Physical quality of water from flood affected and unaffected area

pH

From the table 1, we can observe that pH of water from control area (6.433 ± 0.24) was significantly different from affected areas (5.982 ± 0.05). This finding was in contrast with Okomonda et al. (2013) observed pH of flood affected area in Nigeria shows 6.6 and Mansouri et al. (2013) studied the pH ranges around 8.3 in Birjand flood occurred in Iran. Decreased pH of water in the present study area may be due to organic matter and bacterial contamination which will happen during flood and these contaminations can produce carbon di oxide, it will in turn produce carbonic acid, resulting in the decreased pH of water. So in flood affected area, one of the frontline actions should be to restore the pH of water.

Electrical Conductivity (EC)

EC indicates the total concentration of ionized elements of water (Huq and Alam, 2005). The desirable and permissible value of EC was 400 and 2000 µS/cm respectively at 25ºC (Thambi et al., 2015). From the table 1, it was observed that EC of water from the control area (62.33 ± 2.29) was not significantly different from the other areas (64.50 ± 2.14). This observation was higher when compared with Emeka and Osondu (2016) who described that EC value recorded after flood in Ogunta (28 µS/cm) and Awbana (15 µS/cm). Okomoda et al. (2013) observed the value of EC ranges from 113 to135 µS/cm in Lokaja flood in Nigeria which was higher when compared with present study. Present study shows that EC in general was increased in the both flood affected and unaffected area, this does not significantly vary one place to place because of the huge difference in EC value from place to place within the study area. But this observation is significant enough for evolving animal husbandry management plan to make water profile at Panchayath level.

Total Dissolved Solids (TDS)

From the table 1, we can observe the TDS of water from control area (398.93 ± 14.65) was not significantly different from affected areas (401.73 ± 10.75). The observed reading was in accordance with Deepa Mohan (2019) who reported that TDS of flood water in River Periyar during 2018 flood was 401 mg/L. The TDS change soon after the flood events and hence the long term studies may not reflect transient physical changes like TDS in studies like this.

Turbidity

Turbidity reflects the lack of transparency of water. In drinking water it fluctuates depending upon the amount of organic matter, clay, microscopic organisms and phytoplankton (Mishra, 2005). From the table 1, we can observe the turbidity of water from control area (1.01 ± 0.21) was significantly different from affected areas (0.52 ± 0.05). This observation was in contrast with Mangalekar and Samant (2015) who identified turbidity of water ranges from 1.8 to 5.5 NTU in 2011 flood which occurred in Panchganga river basin. As the samples were taken much after the flood happened in this study which concentrates more on long term effects, the reduced turbidity of flood affected water may be due to the settling effect.

Chemical quality of water from flood affected and unaffected area

Phosphate

Table 2 shows that, there was no significant difference between the control area (0.16 ± 0.02) from the affected area (0.30 ± 0.03) of phosphate in water. The observed value was in accordance with Manjare et al. (2010) who reported the phosphate value during monsoon changes in Tamdalge Tank in Kolhapur District ranges between 0.12 mg/L to 12.38 mg/L. Phosphate normally ranges from 0.005 to 0.05 mg/L in unpolluted water, when this has increased above the level of 0.05 to 0.25 it may cause hyper-trophication and eutrophication. To maintain the low algae growth in well water phosphate level should be less than 0.005 mg/L.  Human health impact was minimal but intestinal problems may arise if ingested with more quantities and it can cause damage to aquatic organisms due to depletion of oxygen which occurs in water body (Merolla, 2012). In the present study the values of phosphate is slightly higher.

Nitrate

Nitrate is one of the contents present in urban as well as rural runoff water which acts as nutrient in water. (Johnston and Dawson, 2005). Table 2 shows that, there was no significant difference between the control areas (6.41 ± 0.52) from the affected areas (6.68 ± 0.13) regarding nitrate in water. This observed value was accordance with Mmom and Aifesehi (2013) they reported all the water samples were within the WHO standard after 2012 flood in Orashi Province of the Niger Delta, Nigeria. Similar finding was recorded by Manjare et al. (2010) in Tamdalge tank in Kolhapur District during climate change.

Hardness

From the table 2 we can observe that hardness of water from control area (20.00 ± 1.15) was not significantly different from affected areas (22.24 ± 0.73). This value was in contrast with Emeka and Osondu (2016) they observed the Hardness value in Oguta and Awbana river flood 2012 in Nigeria was 1 and 2.6 mg/L.

Alkalinity

From the table 2, we can observe that alkalinity of water from control area (320.00 ± 25.82) was significantly different from affected areas (189.71 ± 8.2). Similar finding was done by Emeka and Osondu (2016) says that alkalinity alkalinity value of 196 and 128 mg/L in Oguta and Awbana River flooding in Nigeria 2012. The flood affected area has less value than 200 mg/L which was within the desirable limit drinking water. (Thambi et al., 2015)

Chloride

From the table 2, we can observe that chloride of water from control area (141.40 ± 5.14) was significantly different from affected areas (191.59 ± 7.13). The study area was similar with the findings of Sun et al. (2012) and Emeka and Osondu (2016) were all the samples were within the standard values. The present study reveals the entire sample from flood affected area was within the permissible limit of WHO standard (less than 250 mg/L) even though significantly differ from unaffected area.

Reference

Acharya, S. M. 2018. Collection and preparation of soil, water and plant samples for analysis. Int. J. Chem. Studies. 2: 3298-3303.

American Public Health Association (APHA). (1989). Standard methods for the examination of water and wastewater (17th ed.). Washington, DC, USA.

American Society for Testing and Materials International, 2003. Annual Book of ASTM Standards, Water and Environmental Technology (V. 11.01).West Conshohocken, Pennsylvania. pp. 6-7.

Central Statistical Office. (2018–2019). Agricultural statistics at a glance. Government of India.

Deepa Mohan., Pooja Suresh and Liya Kalappurackal Chackochan. 2019. The Influence of Flood and the Variation in Water Quality Index in River Periyar, Kerala, India. J. Environ. Sci. Toxicol. Food Technol. pp. 2319-2402

Emeka, U. J. and Osondu, N. A. 2016. Post impact studies of the 2012 flooding on the environment and health of Oguta residents, Southeastern, Nigeria. J. Multidisciplinary Engng. Sci. Technol. 11: 28-36

Huq, S. M. I. and Alam, M. D. 2005. A Handbook on Analyses of Soil, Plant, and Water. BACER-DU, University of Dhaka, Dhaka. pp. 1-246

Intergovernmental Panel on Climate Change (IPCC). (2014). Climate change 2014: Fifth assessment report. Cambridge University Press.

Johnston, A. E. and Dawson, C. J. 2005. Phosphorus in agriculture and in relation to water quality. Peterborough: Agricultural Industries Confederation

Joshi, D. M., Kumar, A. and Agrawal, N. 2009. Studies on physicochemical parameters to assess the water quality of river Ganga for drinking purpose in Haridwar district. Rasayan. J.  chem. 1: 195-203.

Maiti, S. K. (2016). Handbook of methods in environmental studies (Vol. 1). ABD Publishers.

Manjare, S. A., Vhanalakar, S. A. and Muley, D. V. 2010.Analysis of water quality using physicochemical parameters Tamdalge tank in Kolhapur district, Maharashtra. Int. J.  Adv. Biotech. Res. 2: 115-119.

Mangalekar, S. B. and Samant. J. S. 2015. Evaluation of water quality of Panchganga River with reference to Water-borne disease. Int. J. Scient. Res.  10p.

Mansouri, B., Moussavi, S. P., Salehi, K., Salehi, J., Kardan-Moghaddam, H., Mahmoodi, M. and Etebari, B. 2013. Assessment of Birjand flood plain water quality by physico-chemical parameters analysis in Iran. J. Adv.  Environ. Hlth. Res. 2: 101-111.

Merolla, S. 2012. The effect of floods and high rainfall on the water quality in selected sub-areas of the upper Vaal Catchment (Doctoral dissertation, University of Johannesburg).

Mishra, P. C. 2005. Some aspects of the quality of water in and around Rourkela (Doctoral dissertation).

Mmom, P. C. and Aifesehi, P. E. 2013. Impact of the 2012 flood on water quality and rural livelihood in the Orashi Province of the Niger Delta, Nigeria. J. Geogr. Geol. 3: 216.

Okomoda, V. T., Solomon, S. G. and Ataguba, G. A. 2013. Physicochemical parameters, microbiological water quality and fish abundance in flooded areas of Lokoja, Nigeria. Asian Fish. Sci. 2: 101-114.

Ramasamy, S. M., Gunasekaran, S., Rajagopal, N., Saravanavel, J. and Kumanan, C. J. 2019. Flood 2018 and the status of reservoir-induced seismicity in Kerala, India. Nat. Hazards. 1: 307-319.

Sun, R., An, D., Lu, W., Shi, Y., Wang, L., Zhang, C., Zhang, P., Qi, H. and Wang, Q. 2016. Impacts of a flash flood on drinking water quality: case study of areas most affected by the 2012 Beijing flood. Heliyon. 2p

Thambi, M., Cherian, T., Jose, A. T. and Jamal, S. 2015. Physico-chemical characteristics of ground water samples from different areas of Thrissur District, Kerala State, India. Int. J. Environ. 2: 182-192.