Heavy Metal Contamination Due to Waste Disposed Zone of Shahbazpur Gas Field in Bangladesh: Environmental and Health Impact Assessment

Abstract

Produced water constitutes the largest fraction of waste generated in the oil and gas extraction industry, while drilling waste represents the second major fraction. Both types of waste are typically discharged into evaporation ponds, which therefore serve as the primary accumulation zones for toxic elements. The study aims to evaluate the extent of elemental contamination associated with hydrocarbon extraction activities at the Shahbazpur gas field in Bangladesh by analyzing elemental abundances in both subsurface reservoir core samples and surface environmental samples (soil and drilling mud). A total of fourteen samples, including three gas reservoir well core samples (depths: 2591.8–3699.0 m) and eleven surface environmental samples (soil and mud from evaporation ponds and surrounding areas), were collected for analysis. In this study, twelve elemental abundances and different environmental contamination indicators were analyzed using the Energy Dispersive X-ray Fluorescence (EDXRF) method. The analysis was conducted on both gas-well core samples and environmental soil and mud-soil samples collected from the Shahbazpur gas field. In the environmental soil and mud-soil samples, Pb concentrations range from 7.3 to 113 ppm, with an average value of 71 ppm, whereas the corresponding Upper Continental Crust (UCC) value is 17 ppm. Similarly, As concentrations range from 10 to 44 ppm, with an average of 27 ppm, compared to the UCC value of 4.8 ppm. These results indicate that the average concentrations of both Pb and As in the soil and mud-soil samples are significantly higher than their respective UCC background values. The calculated enrichment factor (EF), geo-accumulation index (Igeo), and contamination factor (CF) for Pb indicate severe enrichment, moderate geo-accumulation, and considerable contamination in most soil and mud-soil samples.  Overall, the environmental assessment suggests considerable ecological risk, indicating that the soils in the study area are increasingly polluted by toxic elements such as Pb and As introduced through gas production activities. The hazard study of As for children exhibits the existence of chronic non-carcinogenic risk and medium to high cancer risk, though children are more susceptible to heavy metal/metalloid.

Keywords: Environmental contamination, Heavy metal contamination, Evaporation pond, Gas-well core sample, Gas field.

1. Introduction

Over the past two decades, growing population demands and the expansion of industrial processes-such as mineral exploration and oil and gas production-have heightened the focus on assessing environmental contamination hazards, including NORMs (e.g., U-238, Ra-226, Th-232, K-40), organic compounds, mutagenic and carcinogenic substances, toxic heavy metals (e.g., As, Cd), and emerging pollutants like microplastics and nano-materials (Ghanavati et at., 2019; Babaei et al., 2018; Begum et al., 2021). Oil and gas operations pose significant environmental challenges across air, water, soil, and biodiversity, which are interconnected

and collectively influence ecosystem health and human well-being. (Etim et al., 2026). Some of these contaminants exhibit extreme geochemical mobility and some of these contaminants are nephrotoxic, although their toxicity is highly dependent on their chemical form (Wilk et al., 2017; Ebrahimpour et al., 2010). It is often assumed that toxicity and heaviness are interconnected, and even slight exposure  can persuade toxicity (Duffus et al., 2002). Previous studies have shown that industrial waste from coal, gas, and oil exploration industries have elevated the levels of these contaminants in the surrounding environment, posing health risks to workers and local communities (Begum et al., 2022., Al Nabhani et al., 2016; Ahsan et al., 2019). In Bangladesh, a total of 27 gas fields continue to play a critical role as the country's primary energy source (Begum et al., 2022; Hassan Shetol et al.,2019). According to the International Energy Agency, over 70% of global energy demands are currently met by gas and oil (Begum et al., 2022; International Energy Agency 2019).

According to geological studies, organic matter trapped in sedimentary rocks undergoes thermal cracking within subsurface geological formations, leading to the production of hydrocarbons. These sedimentary rocks, known as source rocks, facilitate the conversion of organic matter and kerogen into oil and gas when they mature. Sedimentary basins play a crucial role in the generation, migration, and accumulation of hydrocarbons, forming the foundation of global petroleum systems (Alamu & Dada, 2025). Porous sedimentary rocks, which can hold water, oil, and gas, form hydrocarbon reservoirs when covered by an impermeable layer known as cap rock (Igunnu et al., 2014). Due to the lighter nature of oil and gas compared to water, these hydrocarbons migrate upwards or laterally through small cracks and faults in the carrier rock under pressure. Key examples of source rocks include coal, shales, and carbonates (Jonkers et al., 1997).

When 'formation water' from a gas and oil reservoir is extracted along with hydrocarbons, it is referred to as produced water (Fakhru’l-Razi et al., 2009; Reátegui-Zirena et al., Vaikosen et al., 2014). This water can mobilize more soluble metals in the surrounding environment and often contains elevated levels of contaminants. The characteristics of produced water vary widely depending on factors such as geochemistry, geological age, depth, and the chemical composition of the hydrocarbon-bearing zone. Produced water from oil and gas extraction activities often contains toxic contaminants in varying concentrations, influenced by the age of the hydrocarbon production well and the geological formation of the reservoir. Produced water usually is the wastewater generated in the largest volume during the production of oil and gas wells that are brought to the surface of the earth. Produced water contains a wide variety of chemicals and naturally occurring radioactive elements such as uranium, thorium and potassium that have been dissolved or dispersed from the hydrocarbon and the geologic reservoir containing the formation water (Oghenevovwero & Gregory, 2018).  Fakhru’l-Razi et al. (2009), Utvik (1999), Menzie et al. (1982) reported that the iron content in produced water could be 560 to 2,340 times higher, while barium and manganese could be 20 to 81 times higher, and thallium, cadmium, chromium, strontium, and mercury could be 3 to 17 times higher than levels found in seawater.

The gas and oil industry, despite its significance, is considered one of the major contributors to soil contamination due to the presence of highly significant contaminants (Nazarpour et al., 2019). These contaminants enter the environment through both natural and anthropogenic processes, with human activities often exacerbating their levels. When concentrations exceed normal levels, they become hazardous to the soil, and their bioaccumulation in biota can have detrimental effects on human health through ecosystem interactions (Ogunkunle et al., 2013; Malik et al., 2014). Petroleum pollution negatively affects soil biocenosis, seriously changes the chemical composition, structure and properties of soil, and reduces soil fertility and arable value. Petroleum-polluted soils are not suitable for agricultural and recreational uses and are potential sources of contamination of surface and ground waters (Stepanova et al., 2022).

The majority of waste generated by the gas and oil production industries is recognized as produced water. This waste is a complex mixture of inorganic and organic compounds, including dispersed and dissolved oils, scale products, waxes, radionuclides, dissolved gases, salts, formation solids, treatment chemicals, dissolved oxygen, and microorganisms (Begum et al., 2022; Fakhru’l-Razi et al., 2009; IAEA 2004; API 1992; NRPA 2004; Ray et al., 1992). Fasihi et al. conducted an assessment of environmental contamination in Tehran, revealing high concentrations of petroleum compounds and contaminants in the air and soil, which were found to be harmful to human health (Fasihi et al., 2017; Borojerdnia et al., 2020). The Shahbazpur gas field annually discharges 2,753,244 liters of produced water, containing substantial amounts of scale and sludge (BAPEX,2023).

The second major source of waste is drilling waste produced by the hydrocarbon exploration industry (Ghorbani et.ai., 2020). Drilling wastes are the second largest volume of waste, behind produced water, generated by the oil and gas exploration and production industry. Drilling waste includes drill cuttings, used drill mud, and drilling wastewater (Ilinykh et al., 2023). Drilling muds contain various toxic pollutants, with the types and concentrations of metals and metalloids differing significantly among different types of mud (Ware et al., 1993). Compounds such as arsenic (As), lead (Pb), and cadmium (Cd) are not deliberately added to drilling fluids but can occur as trace contaminants from materials used in the drilling process, such as pipe dope, or from the formation itself (Ware et al., 1993). Pipe dope, which is used to protect drill pipe threads, often contains high concentrations of lead, copper, and zinc (Ghanavati et al., 2019). Barium (Ba), typically in the form of barite (Ba-sulfate), is commonly used to weight drilling fluids. Consequently, the hydrocarbon exploration industry is a significant source of pollutants, mainly introduced through the disposal of produced water and drilling muds.

In onshore hydrocarbon drilling operations, drilling fluids are typically disposed of in on-site reserve pits. Production wastes, such as produced water, are directed to evaporation ponds after being processed through multiple layers of stones and sands in skimming pits, rather than being reinjected. The primary environmental threat from the petroleum industry arises from the migration of dissolved metals from waste disposal sites to groundwater and surface water, as well as the release of vapors and particles into the atmosphere (Ghanavati et al., 2019). In Bangladesh, drilling wastes, including mud, brine, and produced water, are generally deposited into waste evaporation ponds. Consequently, these ponds are considered major repositories for toxic elements (Begum et al., 2021). During the rainy season, these contaminated ponds can overflow, spreading pollutants to nearby ponds and soils, thereby causing environmental contamination in the local pedosphere.

Ghorbani et al. assessed surface soil from the Ahvaz oil field by analyzing 66 soil samples to evaluate potential health hazards from exposure (Ghorbani et al. 2020). Their study found elevated levels of pollutants originating from anthropogenic sources, such as drilling mud and petroleum seepage (Ghorbani et al., 2020; Karbassi et al., 2015) reported that contaminants from soil in major oil fields, like those in Ahvaz, Iran, pose serious human health risks and require measures to control the dispersion of toxins. Atoufi and Lampert highlighted that increased oil and gas production in the United States has led to environmental contamination, particularly associated with the management of produced water and long-term sediment contamination by toxic compounds (Atoufi et al., 2020). They suggested methods for reducing environmental contamination from sediments, including surface covering, phytoremediation, and bioremediation.

This study involves analyzing the elemental abundances in environmental samples from the Shahbazpur gas field in Bangladesh, as well as core samples from gas reservoir wells at depths ranging from 2591.8 to 3699.0 meters. The objective is to assess potential elemental contamination of the surface environment resulting from gas extraction activities. The analysis includes determining the elemental concentrations in both the gas reservoir zones and surface environmental soil and mud samples. Toxic waste elements are identified using the energy dispersive X-ray fluorescence (EDXRF) method. Environmental indices for assessing the ecological risk and assessing both the non-carcinogenic and carcinogenic human health risk from heavy metal/metalloid in soil through ingestion, inhalation of particles and dermic exposure have been conducted.

2. Materials and Methods

2.1 Study Area Profile

The study area falls in the Bengal Foredeep of the Geotectonic province 2, where in the Foredeep region, the stratigraphic succession is of Neogene age starting with the basal Miocene Surma Group (Bhuban and Bokabil; marine to deltaic sequences of clastic rocks; within tidally influenced deltaic depositional environments), which is overlain by the fluvial (lacustrine to overbank), Plio-Pleistocene Tipam Group (sandstone and Girujan Clay), Dupi Tila formations and topmost alluvium respectively (Figure 1C; Table 2) (Begum et al., 2021; Alam et al., 2022). The tectonic and depositional antiquity of the Bengal delta/ Bengal Basin (BB) is interconnected to the development of major Himalayan orogeny (Begum et al., 2021). The basin acts as a land bridge between Himalayas and Bengal deep sea fan. It occupies dominantly by the Meghna-Ganges-Brahmaputra river scheme, where the Surma Group contain subequal proportion of sandstone and shale inter beds (Begum et al., 2021., Uddin et al., 1998; Yang et al., 2020). It is inferred that the sediments in the repository sink were derived likely from continental provenance i.e., recycled orogenic region the Indo-Burman and eastern Himalaya eastern Himalaya Ranges of dominantly felsic parental rocks (granite, rhyolites, para-gneiss and schist, granite-gneissic) (Najman et al., 2012; Rahman et al., 2017; Rahman et al., 2020; Rashid et al., 2023). Climate and tectonic disturbances (Himalayan upheaval event) played a significant role in controlling weathering, erosion and denudation process in the source area (Johnson et al., 1991; Gani et al., 2003; Hossain et al., 2019).

The study zone belongs to Ganges tidal floodplain and Young Meghna estuarine floodplain, two major physiographic units (Brammer et al., 2017). Accretion and erosion are constantly actively taking place on the land margins, continuously altering the shape and morphology of the area. During the rainy season, the deposited soil is thus always being enhanced by heavy silts set down by rivers (high discharge period). It occupies western territory of the Meghna River’s mouth. However, some units have been modified its typical characteristics by post depositional processes such as weathering, denudation, leaching, ploughing, bioturbation activities, etc. The investigated area was studied with satellite images of maps then morphostratigraphic units are defined by field inspection and studied with different GIS tools for conformation. The Holocene sediment covering the mapped zone can be classified into 3 geomorphic map units (fluvio-tidal plain; tidal plain, estuarine bar) based on distinct geomorphic, lithologic, mineralogic, geochemical, granular and sedimentological characteristics (shape, size, aerial extension, sand and clay ratio, association and assemblages). The soils contain olive gray to gray silty loam to silty clay layers (Ahsan et al., 2019). The cultivated topsoil (both calcareous and non-calcareous tidal floodplain grey soil) characteristics; clay (15.6-33.3) %, Fet (1.3-4.3) %, Al, CEC 8.7 (4.5-11.9) cmol/kg, Ct (0.84-1.5) %, EC (0.39-7.5 ) ds/m, pH (5.7-8.3), organics (1.1-1.5) %, N (0.063-0.087) %, Eh, St (3.0-7.3) %, P (0.24-0.68) % and the designated soil series are Bhola, Jhalakati, Ramghati, Nilkamal, Polimati, where mean granulometric distribution is as sand 18.4%, silt 58.2% and clay 23.4% (Uddin et al., 1998). The soils are salty in the region and in reaction, they are neutral to alkaline. These important influential attributes control the background composition of the soil matrix. An increase or decrease of the level of geochemical species in the soils is possibly due to local differences in the parental geological/lithological/mineralogical variations and biogeochemical activities. Bhola (very fertile agricultural soil) is, alluvial island and estuarine mouth bar, mainly of flat with gentle relief, developed by active fluvio-tidal process and filled with approximately 90 m thick Holocene sediments. Elevation ranges from average of 2.9 m (1.7-3.0 m) (north) above mean sea level. However, southern part and newly developed landform (islands, young char land) have an elevation ~1.7 m. Tidal floodplain is presently above highest spring tide level. Average (range) concentrations of SiO2, Al2O3, Fe2O3, Na2O, and K2OAl2O3/TiO2, and chemical index of alteration (CIA) in soil are 65.8 (56.4-78.9), 12.8 (8.2-18.6), 3.6 (1.2-6.8), 0.34 (0.13-1.1), and 2.0 (1.2-2.9) %, (except elemental ratio) respectively (Begum et al., 2021; Ahsan et al., 2019; Uddin et al., 1998). The major oxides with other trace elements are closely connected to the soil mineral verities and soil characteristics and are essentially governed by the (alumino) silicates. The major elements abundances, for instance Si, Al, K-rich phases are chiefly associated to quartz, clays and feldspar, while Fe-Mn (oxides) oxihydroxides and Ti are probably bound to titanite, rutile, anatase, ilmenite in the soils. K2O/Al2O3 relations (ranges from 0.09 to 0.30 with mean: 0.16) for the investigated soils recommend the prevalence of clays (K2O/Al2O3: 0-0.30) rather than feldspars (0.3-0.9) or illites (0.28) (Cox et al., 1995). Although, high K2O/Na2O ratio (ranges from 2.6 to 18.6 with mean: 6.6) demonstrates possible existences of feldspars (Roser et al., 1988).

Figure. 1 (A) Modification of the Bengal Basin Tectonic map after Begum et al., showing different tectonic elements, study location; (B) location and geology of Shahbazpur well#2 and #4 area and collected sample points; (C) lithology and stratigraphy with samples depths of Shahbazpur two wells (Begum et al., 2021).

Furthermore, SiO2/Al2O3 relation (ranges from 3.4 to 8.1 with mean: 5.46) in measured soils are considerably higher than that for kaolinite (1.18) (Roser et al., 1988). Al2O3/TiO2 ratios (ranges from 2.0 to 29.8 with mean: 15.5) for the soil samples found comparatively greater than the mafic (3-8) but lesser than felsic sources, and are comparable with intermediate source rocks (8-21), which point out that the sediments probably resulting from felsic to intermediate igneous rock origin i.e. granitic source (Begum et al., 2021). Some essential major and trace ions are expelled from the sediment matrix depending on their mode of association and geochemical processes. Weathering, erosion, leaching, transformation, dissolution, mobilization, oxidation processes, and dissolution of elements-minerals interacting with ambient environmental conditions are caused. The area experience tropical to subtropical monsoon climate, with a rainy season occupying from June to October. Mean annual rainfall is about 220 cm. The annual temperatures vary from 14.0 to 26.9 ˚C (min) and 25.7 to 33.7 ˚C (max). Humidity remains relatively high throughout the year, 66.7%. The density of population was 542.5 (1153 person in the country) per sq.km. in 2022.

2.1.1 Geological Map and Log Preparation

From the website http://glovis.usgs.gov, Landsat satellite view of 2018 has been used in the current research. Here ArcMap 10.2 and Erdas Imagine (2014) software have been used to demark the different geological units of the area (Fig. 1B) and layer stack of the image respectively and simultaneously ground truth verifying following others (Rashid et al., 2023). The geological map of study area with geology of the area and stratigraphy, lithology of collected core samples are shown decoratively in Figure. 1 (A, B & C). Geological log was prepared using Stratar 5.0.

2. 2 Sampling and Processing

For hazard assessment of Shahbazbur gas field area, elemental abundances analysis of total fourteen samples (gas reservoir well core: three samples and gas field environmental surface soil and mud soil samples: eleven samples) have been conducted.

Table 1. Depiction of the Shahbazbur gas field environmental surface samples

| SL. No | Sample ID | Sample Description (depth) | Location Description | Coordinates |
| --- | --- | --- | --- | --- |
| 1. | EB-1 | Mud soil (0 - 6") | At north-west side of evaporation pond bottom | 22º27′40″ N 90º44′15″ E |
| 2. | EB-2.1 | Surface soil (0 - 6") | Near fresh water pond | 22º28′51″ N 90º45′43″ E |
| 3. | EB-2.2 | Soil sample (6"- 8") | Near fresh water pond | 22º28′51″ N 90º45′43″ E |
| 4. | EB-3.1 | Surface soil (0 - 6") | Bank of fresh water pond | 22º28′51″ N 90º45′43″ E |
| 5. | EB-3.2 | Soil sample (6"- 8") | Bank of fresh water pond | 22º28′51″ N 90º45′43″ E |
| 6. | EB-4.1 | Surface soil (0 - 6") | Near south side of fresh water pond | Not determined |
| 7. | EB-4.2 | Soil sample (6"- 8") | Near south side of fresh water pond | Not determined |
| 8. | EB-5 | Mud soil (0 - 6") | At south-west side of evaporation pond bottom | 22º28′37″ N 90º45′56″ E |
| 9. | EB-6 | Mud soil (0 - 6") | At north-east side of evaporation pond bottom (chemical wastes dumping zone) | 22º27′41″ N 90º44′14″ E |
| 10. | EB-7 | Surface soil (0 - 6") | Outside gas field boundary, at the bank of local cannel | 22º28′15″ N 90º46′01″ E |
| 11. | EB-8 | Surface soil (0 - 6") | Outside gas field boundary, fresh soil | 22º28′51″ N 90º45′43″ E |

Three gas reservoir well core samples of this gas field were collected from store of Chittagong regional office under Bangladesh Petroleum Exploration and Production Company Limited (BAPEX) in 2018 and sampling of surface soil has been conducted on March’2020. Illustration of the surface environmental samples have been displayed in Table 1 and gas reservoir well core samples description have been showed in Table 2. The longitude and latitude of gas reservoir well-2 and well-4 of Shahbazbur gas field are 90°45'07.01" E, 22°28'13.74" N and 90°45'19.626" E, 22°27'52.095" N respectively which lies in the Hatia trough of the southern part of the Bengal basin central deep.

Table 2. Illustration of the Shahbazbur gas reservoir well core samples

| Well No. | Core No. | Sample ID | Depth (m) | Lithol –ogy | Mineralogy (Begum et al., 2021; Alam et al., 2022; Uddin et al., 1998). |
| --- | --- | --- | --- | --- | --- |
| 2 | C-1 | CB-1 | 2591.8 -92 | Shale | Quartz, feldspar, illite, chlorite, smectite, kaolinite, kaolinite-halloysite and kaolinite-smectite mixed layer, dolomite and smectite, and minor to trace fractions of calcite and siderite |
| 4 | C-2 | CB-2 | 3420-21 | Sand | Quartz, feldspar, mica and traces of heavy minerals (e.g., garnet, zircon, tourmaline, epidote, rutile, biotite, kyanite, sillimanite, allanite, hornblende, etc.) |
|  | C-3 | CB-3 | 3698-99 | Shale | Quartz, feldspar, illite, chlorite, smectite, kaolinite, kaolinite-halloysite and kaolinite-smectite mixed layer, dolomite and smectite, and minor to trace fractions of calcite and siderite |

From Shahbazbur gas field area total of eleven samples, where 3 surface (0 - 6") soil samples, 3 sub-surface (6"- 8") soil samples, 3 mud soil of waste deposited evaporation pond bottom and 2 surface (0 - 6") samples outside the gas field boundary have been collected. Produced water falling corner of the evaporation pond, where waste dumped (sample ID: EB-1) is selected as the sample location reference point. Different areas along with sampling locations of the Shahbazbur gas field have been shown in Figure 2.

According to Begum et al. mud-soil samples were collected from the gas field evaporation pond using acrylic pipe screwing towards the mud-soil up to 6" depth and after that, blocking with rubber-cork of the upper open-side, pipe was unscrewing up word. Then hammering from opposite side of the pipe to take out the samples after the cork was removed with taking required protection to avoid cross-contamination (Begum et al., 2022).

| a |  |
| --- | --- |
| b | b |
| c | c |
| Figure 2. (a) Different areas of Shahbazbur gas field; (b) sampling locations, where 2,3,4 and 6 represents EB-2.1, 2.2; EB-3.1, 3.2; EB-4.1, 4.2 and EB-6 respectively; (c) evaporation pond site. |  |

To get constant weight, the samples were dried in electric-oven at 60°C and to acquire homogenized form, samples were powdered using agate mortar. Finally, pellet has been made using ten tons hydraulic pressure (FRITSCH GmbH, Germany).

2.3 Elemental Analysis

Quantification of elemental concentration (K, Ca, Ti, Mn, Fe, Ni, Zn, As, Sr, Y, Zr and Pb) in samples were conducted by Energy Dispersive X-ray Fluorescence (EDXRF) technique, which is a multi-elemental nuclear analytical technique, best suited for solid sample analysis specially soil and mud-soil (Rakib et al., 2021; Jolly et al., 2023; Rahman et al., 2024). Overall set up of the analytical system include a Cd-109 radioisotope annular source to excite the sample, a Si (Li) detector to detect the generated X-rays (Canberra, Model SL 80175), a multi-channel analyzer to display the entire spectrum, a high-voltage power supply and an amplifier to amplify the X-ray (Canberra, Model 2024). Basically, homogeneously powdered each sample with 1 mm thickness, 0.7 cm diameter and in pellet form have been used for the irradiation as a part of elemental analysis in the studied sample. X-ray spectrum intensities of the elements in the sample have been analyzed with the peak fitting deconvolution system software (Khan et al., 2019). Samples examined using two pellets of each sample to represent the data.

2.4 Precision and Accuracy

The precision percentage and accuracy of the EDXRF technique for quality assurance has been conducted by certified reference material (Montana-1) analyzing under the calibration curve which was constructed following (Jolly et al., 2022) using standard material: IAEA/ soil-7. It has been found that the precision percentage varies from 90.6 to 109.1% with the accuracy ranges: (- 9.4 to 9.1) %.

2.5 Environmental Indices for Assessing Ecological Risk

Base-line selection is essential to assess the elemental abundance of soil and mud-soil samples by environmental contamination indicators such as, geo-accumulation index (Igeo), enrichment factor (EF) enrichment factor (EF) and contamination factor (CF). The elemental abundances of upper continental crust (UCC) (Rudnick et al., 2014) have been used in this study as the base-line data alike most of the previous studies.

2.5.1 Enrichment Factor (EF)

EF is used broadly to distinguish manmade and natural sources of metals and metalloids (Birch et al., 2003; Khan et al., 2019). Fe is used in this work to estimate EF as Fe naturally has uniform concentration and exhibits similarly to geochemistry of a lot of trace elements (Ergin et al., 1991; Bhuiyan et al., 2010; Tamim et al., 2016). Equation for EF measurement is as bellow:

EF=Cm∕FⅇSampleCm∕FeBackground

Here, Cm represents concentration of metal. EF above 1.0 suggests as manmade influence, yet again EF values 5 to 10 and above 10 are the warning of severe and very severe alteration of natural elements, respectively (Brich et al., 2008).

2.5.2 Geo-accumulation Index

Igeo is stated by Muller’s equation as below (Brich et al., 2008):

Igeo=log2Cz1.5 X Bz

Where, Cz indicates metal (z) concentration, Bz indicates metal (z) geochemical background concentration, and factor 1.5 is considered as background correction matrix. Igeo consists of several grades (Ma et al., 2016). Where, 0 <Igeo< 1 grade indicates uncontaminated to moderately contaminated and 1 <Igeo< 2, 2 <Igeo< 3, 3 <Igeo< 4, 4<Igeo< 5 grades indicate moderately contaminated, moderately to heavily, heavily, heavily to extremely contaminated correspondingly, again 5 <Igeo grade can be hundredfold greater than base-line, which is open grade and indicates heavily to extremely contaminated (Bhuiyan et al., 2010).

2.5.3 Contamination Factor (CF)

For specific hazardous metal, contamination factor is calculated by the following equation (Hakanson et al., 1980).

CF=CMSampleCMBackground

In which, CM = Concentration of metal. Contamination factor is useful for contamination status detection of hazardous metals and characterized as CF < 1, 1 to 3, 3 to 6 and CF > 6, which referred to as low, moderate, considerable and high (Zhao et al., 2012).

2.6 Assessing Human Health Risk from Heavy Metals in Soil

The risk of non-carcinogenic and carcinogenic heavy metal/metalloid has been assessed by the equations proposed in (Jolly et al., 2022; EPA/540/1-89/002 1989; US EPA 2001). In these assessments for human direct exposure to heavy metal/metalloid from soil, three routes through ingestion, inhalation of particles and dermic

Thus, for non-carcinogenic risk: Hazard Quotient, HQ = ADD/RfD

Here, ADD = the daily dose due to ingestion, inhalation of particles and dermic exposure of heavy metal/metalloid and RfD = the oral reference dose of heavy metal/metalloid.

Again, Carcinogenic Risks: CR = LAAD × SF

Here, LAAD = the lifetime average daily dose due to ingestion, inhalation of particles and dermic exposure of heavy metal/metalloid and SF = the risk factor for cancer of a particular element (Jolly et al., 2022; US EPA 2001; Rahman et al., 2019).

3. Results and Discussion

3.1 Elemental Abundances and Distribution

3.1.1 Shahbazpur Gas Field Environmental Soil

A wide range of dissolved inorganic compounds like anions, cations, heavy metals and radioactive metals could contain in PW that may enter in ecosystem when they are discharged in the soil or in water body (Mayor et al., 2024). Cations such as Sodium (Na), Potassium (K), Calcium (Ca), Magnesium (Mg), Iron (Fe), Barium (Ba) are often found in PW that affect the salinity and scale potentials (Hansen et al., 1994; Neff et al., 2002) showed that the concentration of Ba, Fe, Manganese (Mn), Mercury (Hg) and Zinc (Zn) in PW were 100 to1000 times higher than the surrounding sea water samples. Hg, Lead (Pb) and Arsenic (As) were also found in varied concentrations depending on locations (Neff et al., 2002).67 Most of the studies concentrated on characterization of Mg, Ba, Cadmium (Cd), Chromium (Cr), Pb, Hg, Mn, Ni, Fe, Selenium (Se) and Zn for elemental analysis in the PW samples (Johnson et al., 2008; Igunnu et al., 2014; Camarillo et al., 2018; Spacil et al., 2011; Neff et al., 2011; Salem et al., 2022; Xiao et al., 2021; Hu et al., 2020; Chen et al., 2019). Emmons et. al. studied 32 selective elements in eight different PW samples collected from Permian Basin and Eagle Ford formation in Texas where Mg (400-440 ppm), Se (17-19.5 ppm), Ba (1.6-2.6 ppm) and Zn (1.2-1.6 ppm) were the major constituents along with Pb (0.0005-0.0029 ppm); Vanadium, V (0.009-0.041 ppm); Gallium, Ga (0.054-0.940 ppm); Cr (0.0004-0.024 ppm); Rubidium, Rb (0.080-0.640 ppm) and Cesium, Cs (0.036-0.227 ppm) as a versatile presence in all tested samples (Emmons et al., 2022).  Herein, we have presented the examining data of the total twelve inorganic elements (K, Ca, Titanium (Ti), Mn, Fe, Nickel (Ni), Zn, As, Strontium (Sr), Yttrium (Y), Zirconium (Zr) and Pb) contained in Shahbazpur gas field environmental sediment and soil samples exposed to produced water contaminants that have been presented in Table 3 with mean abundances, relative standard deviation and relative standard data of relevant elements. Again Figure 3 illustrates the spatial distributions maps of As, Ti and Pb elements respectively of the Shahbazpur gas field environmental soil and mud-soil samples.

Table 3. Elemental abundances of Shahbazpur gas field environmental soil and mud-soil samples along with gas well core samples

| Element | Unit | Gas Field Environmental Soil & mud-soil Samples |  |  |  |  |  |  |  |  |  |  |  | Gas-well Core Samples |  |  | Uncertainty* |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | EB-1 | EB-2.1 | EB-2.2 | EB-3.1 | EB-3.2 | EB-4.1 | EB-4.2 | EB-5 | EB-6 | EB-7 | EB-8 |  | CB-1 | CB-2 | CB-3 |  |
| K | [%] | 0.91 | 0.95 | 0.91 | 0.43 | 0.57 | 0.84 | 0.27 | 1.05 | 1.17 | 0.87 | 1.25 |  | 1.15 | 0.86 | 1.08 | 6-8% |
| Ca | [%] | 3.34 | 3.74 | 4.73 | 2.06 | 2.22 | 3.72 | 2.52 | 3.68 | 5.03 | 4.03 | 1.69 |  | 4.98 | 3.08 | 3.74 | 12-14% |
| Ti | [%] | 0.46 | 0.57 | 0.39 | 0.59 | 0.27 | 0.47 | 0.50 | 0.52 | 3.44 | 0.50 | 0.51 |  | 0.90 | 0.50 | 0.50 | 2-4% |
| Mn | [µg/g] | 544 | 732 | 653 | 299 | 503 | 601 | 615 | 568 | 844 | 618 | 639 |  | 786 | 653 | 590 | 5-7% |
| Fe | [%] | 3.93 | 4.48 | 4.16 | 4.76 | 4.74 | 4.74 | 4.76 | 4.42 | 1.52 | 2.88 | 5.14 |  | 10.1 | 4.5 | 6.3 | 3-5% |
| Ni | [µg/g] | 70.5 | 75.5 | 98.4 | 27.2 | 50.1 | 70.5 | 31.9 | 102 | 83.5 | 74.6 | 24.5 |  | 82.5 | 82.0 | 73.3 | 7-9% |
| Zn | [µg/g] | 117 | 79.8 | 116 | 64.0 | 129 | 121 | 102 | 80.0 | 104 | 97.0 | 112 |  | 276 | 102 | 137 | 6-8% |
| As | [µg/g] | 33.2 | 31.8 | 35.6 | 12.1 | 18.4 | 33.2 | 10.7 | 31.9 | 31.9 | 43.9 | 9.83 |  | 26 | 32 | 33 | 3-5% |
| Sr | [µg/g] | 158 | 121 | 152 | 130 | 137 | 122 | 161 | 135 | 391 | 156 | 118 |  | 247 | 127 | 147 | 13-15% |
| Y | [µg/g] | 39.4 | 51.7 | 47.8 | 30.4 | 41.4 | 37.5 | 51.1 | 31.7 | 24.3 | 34.3 | 37.5 |  | 67.1 | 37.0 | 51.0 | 9-11% |
| Zr | [µg/g] | 300 | 351 | 322 | 256 | 248 | 297 | 281 | 238 | 112 | 260 | 268 |  | 490 | 287 | 291 | 18-24% |
| Pb | [µg/g] | 87.1 | 86.8 | 97.1 | 53.3 | 59.4 | 96.3 | 35.1 | 87.1 | 82.5 | 113 | 22.9 |  | 105 | 82.3 | 87.3 | 9-12% |

|  |  | Gas Field Environmental Soil & mud-soil Samples |  |  |  |  |  | Gas-well Core Samples |  |  |  |  | UCCa | World Soil Medianb | Shaleb |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Element | Unit | Mean | SD | RSD | Min. | Max. |  | Mean | SD | RSD | Min. | Max. |  |  |  |
| K | [%] | 0.84 | 0.30 | 36.1 | 0.27 | 1.25 |  | 1.03 | 0.15 | 14.4 | 0.86 | 1.15 | 2.32 | 1.4 | 2.45 |
| Ca | [%] | 3.34 | 1.09 | 32.8 | 1.69 | 5.03 |  | 3.93 | 0.96 | 24.4 | 3.08 | 4.98 | 2.6 | 1.5 | 1.6 |
| Ti | [%] | 0.74 | 0.90 | 121 | 0.27 | 3.44 |  | 0.63 | 0.23 | 36.1 | 0.50 | 0.90 | 0.38 | 0.5 | 0.46 |
| Mn | [µg/g] | 601 | 136 | 22.7 | 299 | 844 |  | 676 | 100 | 14.8 | 590 | 786 | 775 | 1000 | 850 |
| Fe | [%] | 4.14 | 1.06 | 25.5 | 1.52 | 5.14 |  | 6.97 | 2.89 | 41.5 | 4.48 | 10.1 | 3.92 | 4 | 4.8 |
| Ni | [µg/g] | 64.4 | 27.3 | 42.4 | 24.5 | 102 |  | 79.3 | 5.15 | 6.50 | 73.3 | 82.5 | 47 | 50 | 68 |
| Zn | [µg/g] | 102 | 20.2 | 19.8 | 64.0 | 129 |  | 171 | 92.0 | 53.8 | 102 | 276 | 67 | 90 | 120 |
| As | [µg/g] | 26.6 | 11.7 | 43.9 | 9.83 | 43.9 |  | 30.4 | 3.82 | 12.6 | 26.0 | 33.2 | 4.8 | 6 | 13 |
| Sr | [µg/g] | 162 | 77.5 | 47.9 | 118 | 391 |  | 173 | 64.3 | 37.0 | 127 | 247 | 320 | 250 | 300 |
| Y | [µg/g] | 38.8 | 8.73 | 22.5 | 24.3 | 51.7 |  | 51.7 | 15.1 | 29.1 | 37.0 | 67.1 | 21 | 40 | 41 |
| Zr | [µg/g] | 267 | 61.2 | 23.0 | 112 | 351 |  | 356 | 116 | 32.6 | 287 | 490 | 193 | 400 | 160 |
| Pb | [µg/g] | 74.6 | 28.1 | 37.7 | 22.92 | 113 |  | 91.5 | 12.0 | 13.1 | 82.3 | 105 | 17 | 35 | 23 |

* Analytical uncertainty (%) are due to counting statistics and procedure blank correction.  a(Rudnick et al., 2014), b(Bowen et al., 1979).

Figure 3. Spatial distribution maps of As, Ti and Pb elements in the Shahbazpur gas field environmental soil and mud-soil samples (generated using ArcMap 10.2 with IDW Interpolation Method).

In this gas field, concentration of heavy metalloid, As varies from 9.83 ppm to 43.9 ppm with an average of 26.6 ppm and concentration of heavy metal, Pb varies from 22.92 ppm to 113 ppm with an average of 74.6 ppm in the soil and mud-soil samples, where average concentration of As is above five times higher and average concentration of Pb is above four times higher compared to corresponding UCC values. According to Ghorbani et al. [28], for 66 soil samples of Ahvaz oil field, Iran; concentration of heavy metalloid, As varies from (2-28) ppm with an average of 6 ppm and concentration of heavy metal, Pb varies from (8-295) ppm with an average of 67 ppm in the soil, which findings are comparable as soil samples of hydrocarbon production field with our study and some variations due to different geological formation, hydrocarbon production capacity etc (Borojerdnia et al., 2020). Maximum concentration of both As and Pb have been found in the same soil sample (sample ID: EB-7), which location is outside the gas field boundary at the bank of the local canal but connected with the gas field through drainage system. Again minimum concentration of both As and Pb have been found in the same soil sample (sample ID: EB-8), which location is also outside the gas field boundary and collected as baseline data. For Ca, Ti, Mn and Sr elemental abundances have been found highest in sample MB-6 compared to other environmental samples of Shahbazpur gas field area of this study. According to Begum et al. there may have possibility of Ti-rich heavy minerals abundances such as rutile in this gas well core and so, Pb is enriched to soils by suitable soil pH level in the sediments. As ∼95% of the adsorbed Pb is transferable from rutile when soil pH 7 and the Ganges tidal delta plains sediments have pH ∼7(Begum et al., 2021; Rashid et al., 2023). Again lowest elemental abundances have been found for Fe, Y and Zr in this specific sample. Evidence of the mentioned situation could be expressed by constant sum effect of geochemical data for soil and mud-soil, because in a dataset, changes may occur in relative elemental abundances when concentration of one element changes (Hossain et al., 2019). Rashid et al. stated that chlorite is one of the dominant clay minerals of the sediments in BB, again in these sediments, Fe oxyhydroxides are the key hosts of As and also the probable sources of As in sediment are mica and clay minerals (Rashid et al., 2023). Yang et al. affirmed that chlorite acting the role for extraction of As from the sediments (Yang et al., 2016).

3.1.2 Gas Reservoir Well Core Samples of Shahbazpur Gas Field

For the gas reservoir well core samples of Shahbazpur gas field, total twelve elemental abundances (K, Ca, Ti, Mn, Fe, Ni, Zn, As, Sr, Y, Zr and Pb have been analyzed and presented in Table 3 with relative literature data of relevant elements.

According to gas-well core data, concentration of heavy metal, Pb varies from 82.3 ppm to 105 ppm with an average 91.5 ppm, where corresponding UCC value is 17 ppm i.e. average Pb concentration is more than five times higher than respective UCC value and also four times higher than corresponding shale value of 23 ppm. Concentration of heavy metalloid, As varies from 26 ppm to 33 ppm with an average 30.4 ppm and corresponding UCC value is 4.8 ppm, where average As concentration is more than six times higher than respective UCC value, also five times higher than corresponding world soil median value of 13 ppm. Ti concentration varies from 0.5 % to 0.9 % with an average 0.63 % and corresponding UCC value is 0.38 %. Ti-rich heavy minerals like rutile may be present in this gas well core samples. So it can be assumed that day by day the soil and mud-soil can be polluted more from gas abstraction activities by anthropogenic incorporation.

3.2. Assessment of Environmental Pollution

For the base-line data, the elemental abundances of UCC (Rudnick et al., 2014) has been selected in this study as selection of base-line data is essential to measure different environmental contamination indices for the soil and mud-soil samples.

3.2.1 Enrichment Factor (EF)

EF in definite soil and mud-soil samples for each element has been presented graphically in Figure 4 to differentiate anthropogenic and natural sources of metals and metalloids. Here, Fe has been used for EF calculation as naturally Fe has just about uniform concentrations and exhibitions identical geochemistry of many trace elements (Ergin et al., 1991; Bhuiyan et al., 2010; Tamim et al., 2016). Enrichment factor values of Shahbazpur gas field environmental soil and mud-soil samples along with gas well core samples are given in Appendix Table A1.

Gas field environmental mud-soil sample, EB-6 of the north-east side of the evaporation pond where chemicals (mud, brine) are also dumped along with produced waste water, pointedly higher EF of all the assessed metals and metalloids were found related to other soil and mud-soil samples of Shahbazpur gas field area of this research work.

Before the research result obtained, it had been predicted that relatively greater pollution could be observed in the bottom mud-soil samples of waste released evaporation pond, principally in the sample: EB-6 of north-east pond side, where chemicals (mud, brine) be released also. Again relatively minor contamination could be observed in soil samples away from this waste releasing zone (evaporation pond area) for metal/metalloid. Interestingly the outcome has been acquired precisely correct as predicted. The As EF value varies from 1.6 to 17 with the maximum value, 17 in EB-6 sample. Alike trends are found for Pb (i.e. EF varies from 1.03 to 12.5) and Ti (i.e. EF varies from 1.02 to 23.3) with the maximum EF values for both Pb and Ti have been observed in same EB-6 sample. Saleh et al showed that the EF of heavy metals and rare earth metals in soil samples around the oil and gas exploration areas where they found Co and Cr were moderately severe enriched, As was extremely severe enriched (EF>50) and Zn, Zr, Cu and Pd were categorized severe enriched with the EF values 10.9, 13.2, 14.3 and

Figure 4. Plotted enrichment factor of elements in the Shahbazpur gas field environmental soil and mud-soil samples along with gas well core samples.

Figure 5. Plotted geo-accumulation of elements in the Shahbazpur gas field environmental soil and mud-soil samples along with gas well core samples.

20.4 respectively (Saleh et al., 2022). In gas field pipeline material, Ti has been used as one of the elements. Again from gas abstraction process, Ti enrichment can be aroused. Seven samples within eleven soil and mud-soil samples revealed that the samples are around severely enriched with these heavy metal/metalloid and relatively lower EF values were obtained in those samples, far away from pond of waste released, which can be considered as an indication of contamination. Again lowermost EF values of both As and Pb have been observed in EB-8 sample, which was comparatively high land outside the gas field periphery and collected as base-line data idea, which may flourish the accuracy of the data. After this location, both As and Pb EF values are observed greater in EB-7 sample, which location is linked through gas field drainage system though situated outside the gas field boundary, beside a local canal. All the above mentioned evidences disclose the anthropogenic origin of these elements.

3.2.2 Igeo of Elements of Gas Field Samples

Depending on the choice of background levels, Igeo values exhibit huge variations, where mineralogically and texturally comparable elemental abundances of UCC have been selected in this work as the base-line data to measure geo-accumulation for identifying polluted and unpolluted soil and mud-soil samples (Rudnick et al., 2014; Rubio et al., 2000). Geo-accumulation index values of Shahbazpur gas field environmental soil and mud-soil samples along with gas well core samples are tabulated in Appendix Table A2.

The geo-accumulation index (Igeo) of elemental abundances of Shahbazpur gas field environmental soil and mud-soil samples have been presented graphically in Figure 5.

Based on Müller’s Igeo classification, graphical representation in Figure 5 revealed moderately contamination of Pb in all soil and mud-soil samples of the gas field area excluding two samples. One is 6ʺ to 8ʺ depth subsurface soil and another is collected for background information, (sample: EB-8), which may flourish the accuracy of the data.

Figure 6. Plotted contamination factor of elements in the Shahbazpur gas field environmental soil and mud-soil samples along with gas well core samples.

The Igeo value of Pb for EB-8 is -0.15 i.e. without polluted area has been found as expected during collection of the sample. Moderate to heavily contamination of As have been observed in seventy percent of soil and mud-soil samples of this gas field area. Also the Igeo value (0.45) for As of EB-8 is observed lowest, which may be an evidence of the accuracy of the data. Maximum Igeo values of both As and Pb have been observed in EB-7, which location is linked through gas field drainage system though located outside the gas field periphery. According to Ghorbani et al. for 66 soil samples of Ahvaz oil field, Iran; maximum Igeo value of both the heavy metal/metalloid are excellently comparable with our study data (Ghorbani et al., 2020). In EB-6 location i.e. the north-east side of evaporation pond where chemicals (mud, brine) are also discarded along with produced water waste, extremely greater (Igeo value: 2.6) geo-accumulation relative to all other samples has been observed for Ti. Above presented all the evidence disclose the anthropogenic origin of these elements.

The Igeo values for Ca, Ni, Zn and Y in some environmental samples are observed higher than zero i.e., these elements are possibly to be from the manmade source.

In all the samples, the Igeo negative values for K, Mn, Fe and Sr indicate that there is no pollution exist of these elements in the soil and mud-soil samples of the investigated locations as lower than their corresponding natural background levels are found.

3.2.3 Contamination Factor (CF)

To detect the contamination status of metals, the CF of elemental abundances of this gas field environmental soil and mud-soil samples have been graphically presented in Figure 6. Contamination factor values of Shahbazpur gas field environmental soil and mud-soil samples along with gas well core samples have been given in Appendix Table A3.

For both As and Pb, CF values (greater than 6) are observed high in EB-6 and EB-7 samples. Here mud-soil sample EB-6 from evaporation pond north-east side, where chemicals (mud, brine) are discarded along with produced water. Again EB-7 is linked through gas field drainage system though located outside the gas field boundary. According to CF values of As metalloid, 70% of total environmental samples have been highly contaminated. In the same way, 70% of total soil and mud-soil samples have been considerably to around highly contaminated by Pb according to the CF values. Relatively lower contamination have been observed in the samples distant from evaporation pond. And lowest CF values for both As and Pb have been observed in EB-8 from outside the gas-field periphery for base-line information. All the bring up evidences disclose that contamination by these two heavy metal/metalloid are from the manmade sources. Along with these heavy metal/metalloid, for Ca, Ni, Zn, Y and Zr the CF values have been found much higher than one i.e., these elements are possibly from the anthropogenic source.

3.3 Core and Surface Samples Comparison

Almost similar trend is observed for the three environmental indices for core and surface samples data of Shabazpur gas field. Gas reservoir well core samples except CB-1 (which is from another well i.e. well-2 and others from well-4) and also in most of the surface samples are around severely enriched with As and moderate to severely enriched with Pb, where exception in CB-1 could be owing to geological difference of formation. The geo-accumulation index (Igeo) values reveal that, both core and most of the surface samples are moderately polluted by Pb and moderate to heavily polluted by As. Enrichment of elemental abundances relative to corresponding UCC i.e. contamination factor values in gas reservoir well core samples (except CB-1for Pb) and also in most of the surface samples are high and considerable for As and Pb respectively, where contamination factor values of As and Pb are found considerable and high respectively for CB-1. Contamination factor values of Zn and Y, also found considerable in CB-1, where these changes could be owing to geological difference of formation as CB-1 is from another well i.e. well-2 and others from well-4. Consequently, there is a chance of gas field environmental soil to be contaminated more by heavy metal/metalloids like As and Pb from production activities of the gas field.

3.4. Human Health Risk from Heavy Metals in Gas Field Soil

Health risk assessment for mainly contaminated heavy metal/metalloid, As and Pb of Shahbazpur gas field environmental soil to human through all three pathways both for children and adults have been showed in Table 4. The maximum and average values of HQ in pathways both for children and adults have been found as, ingestion > dermal contact > inhalation correspondingly. In Table 4, all factor levels in children are higher than in adults. According to Ghorbani et al. and Qing et al. for CR, our results are in accordance with their findings. In both the age groups, the maximum HQ and CR observed to both As and Pb taken through ingestion (Ghorbani et al, 2020; Qing et al., 2015). According to Tao et al. the exposure risk to heavy metal/metalloid in children through ingestion is higher than exposure risk through inhalation or dermal contact, which is consistent to our results (Tao et al., 2015).

The maximum value of total HQ for all three pathways of As for children is greater than one in Shahbazpur gas field environmental surface soil of our study according to Table 4, which indicates chronic non-cancer hazard is seriously possible. Otherwise, the total HQ of all three pathways of heavy metal/metalloid has been found less than 1 for all cases of adults and most of the cases of children, so non-significant non-cancer adverse effects have been associated on humans in this gas field, as established by Liu et al. and Wei et al. This suggests that the threat of heavy metal/metalloid, in comparison to adults, children are more exposed (Liu et al., 2013; Wei et al., 2015). The order of both HQ and CR have been found in the order: As > Pb both for children and adults, which result is similar for children with the data for soil of Ahvaz oil field (Ghorbani et al., 2020).

Table 4. Health risk assessment factors for heavy metal/metalloid of Shahbazpur gas field environmental soil to human pathway.

| Risk Indices | Population | As |  | Pb |  |  | As |  | Pb |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Core Sample |  |  |  |  | Surface Sample |  |  |  |
|  |  | Mean | Max | Mean | Max |  | Mean | Max | Mean | Max |
| HQing | Child | 6.66 × 10-1 | 7.28 × 10-1 | 1.72× 10-1 | 1.97 × 10-1 |  | 5.74 × 10-1 | 9.62 × 10-1 | 1.40 × 10-1 | 2.12 × 10-1 |
|  | Adult | 7.14 × 10-2 | 7.80 × 10-2 | 1.84 × 10-2 | 2.11 × 10-2 |  | 6.15 × 10-2 | 1.03 × 10-1 | 1.50 × 10-2 | 2.27 × 10-2 |
| HQinh | Child | 1.80 × 10-5 | 1.97 × 10-5 | 4.78× 10-6 | 5.48× 10-6 |  | 1.55 × 10-5 | 2.60 × 10-5 | 3.89 × 10-6 | 5.90 × 10-6 |
|  | Adult | 1.02 × 10-5 | 1.11 × 10-5 | 2.69 × 10-6 | 3.09 × 10-6 |  | 8.76 × 10-6 | 1.47 × 10-5 | 2.20 × 10-6 | 3.33 × 10-6 |
| HQderm | Child | 5.61 × 10-2 | 6.12 × 10-2 | 1.32 × 10-3 | 1.51 × 10-3 |  | 4.83 × 10-2 | 8.10 × 10-2 | 1.07 × 10-3 | 1.63 × 10-3 |
|  | Adult | 7.84 × 10-3 | 8.57 × 10-3 | 1.84 × 10-4 | 2.12 × 10-4 |  | 6.76 × 10-3 | 1.13 × 10-2 | 1.50 × 10-4 | 2.28 × 10-4 |
| HQ | Child | 7.22 × 10-1 | 7.89 × 10-1 | 1.73 × 10-1 | 1.99 × 10-1 |  | 6.23 × 10-1 | 1.04 | 1.41 × 10-1 | 2.14 × 10-1 |
|  | Adult | 7.92 × 10-2 | 8.65 × 10-2 | 1.86 × 10-2 | 2.14 × 10-2 |  | 6.83 × 10-2 | 1.14 × 10-1 | 1.52 × 10-2 | 2.30 × 10-2 |
| CR | Child | 3.25 × 10-4 | 3.55 × 10-4 | 5.41× 10-6 | 6.21 × 10-6 |  | 2.80 × 10-4 | 4.69 × 10-4 | 4.41 × 10-6 | 6.69× 10-6 |
|  | Adult | 3.57 × 10-5 | 3.89 × 10-5 | 5.80 × 10-7 | 6.66 × 10-7 |  | 3.07 × 10-5 | 5.14 × 10-5 | 4.73 × 10-7 | 7.16× 10-7 |

The CR of Pb for both adults and children are around 1×10-6, representing that the CR of Pb in surface soil can be ignored. On the other hand, for children, the CR of As are within level-V (CR value within 1×10-4 to 5×10-4) i.e. in medium to high risk level in our study area according to Li et al and Orosun (Jolly et al., 2022; Li et al., 2017; Orosun et al., 2021). Again for adults, the CR of As are within level III (CR value within 1×10-5 to 5×10-5) to IV (CR value within 5×10-5 to 1×10-4) i.e. in low-medium risk to medium risk levels. So the cancer risk of As invokes rapid attention to human health risks of heavy metal/metalloid as children are more vulnerable.

4. Summary and Conclusion

In this present study we tried to evaluate the presence of the inorganic contaminants only in the Shahbazpur gas field region. Concerning the impacts on environment and human health, pollutants existing in the gas field environmental soil through produced water are potentially poisonous and carcinogenic and can disrupt the endocrine system. In the Shabazpur gas field environmental soil and mud-soil samples, the Pb average concentration is 71 ppm (ranges: 7.3 to 113 ppm), where the UCC (upper continental crust) concentration and world soil median values of Pb are 17 ppm and 35 ppm correspondingly. The As concentration ranges from 10 to 44 ppm with mean value 27 ppm, and the UCC concentration and world soil median values of As are 4.8 ppm and 6 ppm correspondingly. Both As and Pb mean concentration in soil of this gas field are noticeably greater than respective world soil median and UCC values.

In the Shabazpur gas field environmental soil and mud-soil samples, enrichment index for assessing all the elemental contents are got relatively greater values in two samples, where one from chemicals (mud, brine) discarded north-east side of waste-water dumped evaporation pond and another from outside the gas field boundary but linked through gas field drainage. In distant soil samples from evaporation pond, relatively lower values have been observed for most of the elementals. Lowermost values of many elements, like As, Pb have been observed in the sample (outside the gas field periphery) collected for base-line information.

The geo-accumulation index discovered moderate to heavy As contamination and moderate Pb contamination in most of the soil and mud-soil samples. The enrichment factor values in maximum samples of this gas field shown that the soil and mud-soil have been around severely enriched by heavy metals/metalloids, As and Pb. Again contamination factor values are observed high and considerable for As and Pb correspondingly in most of the samples. All the stated evidences disclose that these elements are from the anthropogenic source. In evaluating the whole work, we did not examine the interactions of the constituents presents in produced water or in formation water for identifying the inorganic pollutants.

In this subjected gas field environmental soil to human health risk both for cancer and non-carcinogenic hazard by direct exposure to mainly contaminated heavy metal/metalloid, As and Pb have been assessed. From As, the hazard study for children concluded the existence of chronic non-carcinogenic risk and level V i.e. medium to high cancer risk level in our study area though children are more susceptible to heavy metal/metalloid.

Recommendations

All the environmental indics for assessing the elemental contents recommend substantial pollution of As and Pb in maximum samples of this gas field and may be polluted more through gas production activities. The yearly waste water removal of Shabazpur gas field is 2753244 L along with creating more sludge and scale and day by day the leftover increases. So, this is peak time to be conscious of preventing additional pollution from chemically poisonous elements and taking essential counter to contamination.  Arsenic exposures have emphatically greater cancer mortality rates of liver, skin and kidney and capable of interrupts almost all organs like respiratory, cardiovascular, nervous, gastrointestinal, hepatobiliary systems. Once more, the maximum percentage of lead (Pb) is received into a human being’s kidney; afterward liver, brain, heart soft tissues and also skeleton being attacked and Pb is able to transfer from the exposed pregnant mother to the developing fetus.  Hence, consciousness is essential against more environmental pollution from harmful elements like heavy metals/metalloids.

Future Studies

- Additional research can be conducted to evaluate heavy metals/metalloids Plant Transfer Factor (PTF) of gas field regions.

- Assessment of water sample from pond or local canal of gas field areas for environmental pollution from heavy metals/metalloids.

- Both drilling core and gas field environmental samples of others in Bangladesh have to be considered for further analysis.

- Produced water contamination factors could be assessed in the future to understand the chemistry of the whole contaminants present in the subjected pollution area.

Disclaimer (Artificial intelligence)

Author(s) hereby declare that generative AI technologies such as Large Language Models, etc. have been used during the writing or editing of manuscripts. This explanation will include the name, version, model, and source of the generative AI technology and as well as all input prompts provided to the generative AI technology

Details of the AI usage are given below:

Generative AI model was used only for the grammatical correction. The authors take full responsibility for the content and interpretation of the manuscript.

References

Ahsan, M. A., Satter, F., Siddique, M. A. B., Akbor, M. A., Ahmed, S., Shajahan, M., et al. (2019). Chemical and physicochemical characterization of effluents from the tanning and textile industries in Bangladesh using a multivariate statistical approach. Environmental Monitoring and Assessment, 191(9).

Al Nabhani, K., Khan, F., & Yang, M. (2016). Technologically enhanced naturally occurring radioactive materials in oil and gas production: A silent killer. Process Safety and Environmental Protection, 99, 237–247.

Alam, K., Abdullatif, O., El-Husseiny, A., & Babalola, L. (2022). Depositional and diagenetic controls on reservoir heterogeneity and quality of the Bhuban Formation, Neogene Surma Group, Srikail Gas Field, Bengal Basin, Bangladesh. Journal of Asian Earth Sciences, 223, 104985. https://doi.org/10.1016/j.jseaes.2021.104985

API (American Petroleum Institute). (1992). Bulletin on management of naturally occurring radioactive materials (NORM) in oil and gas production, API Bulletin. E2, Washington, DC.

Atoufi, D., & Lampert, D. J. (2020). Impacts of oil and gas production on contaminant levels in sediments. Current Pollution Reports, 6, 43–53.

Babaei, H., Ghanavati, N., & Nazarpour, A. (2018). Contamination level of mercury in the street dust of Ahvaz City and its spatial distribution. Journal of Water and Soil Science, 22(3).

BAPEX (Bangladesh Petroleum Exploration and Production Company Ltd.). 2023. Annual Report Production division, 2021-22 fiscal year, BAPEX.

Begum, M., Khan, R., Roy, D. K., Habib, M. A., Rashid, M. B., Naher, K., et al. (2021). Geochemical characterization of Miocene core sediments from Shahbazpur gas-wells (Bangladesh) in terms of elemental abundances by instrumental neutron activation analysis. Journal of Radioanalytical and Nuclear Chemistry, 329(3), 239–252. https://doi.org/10.1007/s10967-021-07770-4

Begum, M., Khan, R., Hossain, S. M., & Al Mamun, S. M. M. (2022). Redistribution of NORMs in and around a gas field (Shahbazpur, Bangladesh): Radiological risk assessment. Journal of Radioanalytical and Nuclear Chemistry, 331(1), 317–330. https://doi.org/10.1007/s10967-021-08107-x

Birch, G. (2003). A scheme for assessing human impacts on coastal aquatic environments using sediments. In C. D. Woodcoffe & R. A. Furness (Eds.), Wollongong University Papers in Center for Maritime Policy 14 Coastal GIS. Australia.

Brich, G. F., Olmos, M. A. (2008). Sediment-bound heavy metals as indicators of human influence and biological risk in coastal water bodies. ICES J. Mar. Sci. 65, 1407–1413.

Borojerdnia, A., Rozbahani, M. M., Nazarpour, A., Ghanavati, N., & Payandeh, K. (2020). Application of exploratory and spatial data analysis (SDA), singularity matrix analysis, and fractal models to delineate background of potentially toxic elements: A case study of Ahvaz, SW Iran. Science of the Total Environment, 140103.

Bhuiyan, M. A. H., Parvez, L., Islam, M. A., Dampare, S. B., Suzuki, S. (2010). Heavy metal
pollution of coal mine-affected agricultural soils in the northern part of Bangladesh.
J. Hazard. Mater, 173, 384–392.

Bowen, H. J. M. (1979). Environmental chemistry of the elements. Academic Press.

Brammer, H. (2017). Bangladesh’s diverse and complex physical geography: Implications for agricultural development. International Journal of Environmental Studies, 74(1), 1–27.

Camarillo, M. K., & Stringfellow, W. T. (2018). Biological treatment of oil and gas produced water: A review and meta-analysis. Clean Technologies and Environmental Policy, 20(6), 1127–1146. https://doi.org/10.1007/s10098-018-1564-9

Chen, M., Shafer-Peltier, K., Veisi, M., Randtke, S., & Peltier, E. (2019). Complexation and precipitation of scale-forming cations in oilfield produced water with polyelectrolytes. Separation and Purification Technology, 222, 1–10. https://doi.org/10.1016/j.seppur.2019.04.013

Cox, R., Lowe, D. R., & Cullers, R. L. (1995). The influence of sediment recycling and basement composition on evolution of mudrock chemistry in the southwestern United States. Geochimica et Cosmochimica Acta, 59(14), 2919–2940. https://doi.org/10.1016/0016-7037(95)00185-9

Duffus, J. H. (2002). Heavy metals—A meaningless term? Pure and Applied Chemistry, 74, 793–807.

Ebrahimpour, M., Mosavisefat, M., & Mohabbati, R. (2010). Acute toxicity bioassay of mercuric chloride in an alien fish from a river. Toxicological & Environmental Chemistry, 92(1), 169–173.

Emmons, R. V., Shyam-Sunder, G. S., Liden, T., et al. (2022). Unraveling the complex composition of produced water by specialized extraction methodologies. Environmental Science & Technology, 56(4), 2334–2444. https://doi.org/10.1021/acs.est.1c05826

EPA/540/1-89/002. (1989).  Risk Assessment Guidance for Superfund. Volume I: Human health evaluation manual (Part A). Interim Final; US EPA. U.S. Environmental Protection Agency, Office of Emergency and Remedial Response. Washington, DC, USA.

Ergin, M., Saydam, C., Basturk, O., Erdem, E., & Yoruk, R. (1991). Heavy metal concentrations in surface sediments from two coastal inlets (Golden Horn Estuary and Izmit Bay) of the northeastern Sea of Marmara. Chemical Geology, 91, 269–285.

Fakhru’l-Razi, A., Pendashteh, A., Abdullah, L. C., Biak, D. R. A., Madaeni, S. S., & Abidin, Z. Z. (2009). Review of technologies for oil and gas produced water treatment. Journal of Hazardous Materials, 170, 530–551.

Fasihi, H., Hamidi, M., & Ostadfarag, S. (2017). Investigation of heavy metals and hydrocarbons contamination in Baghershar, Tehran, Iran. Environmental Monitoring and Assessment, 189, 19–32.

Gani, M. R., & Alam, M. M. (2003). Sedimentation and basin-fill history of the Neogene clastic succession exposed in the southeastern fold belt of the Bengal Basin, Bangladesh: A high-resolution sequence stratigraphic approach. Sedimentary Geology, 155(3–4), 227–270.

Ghanavati, N., Nazarpour, A., & Watts, M. J. (2019). Status, source, ecological and health risk assessment of toxic metals and polycyclic aromatic hydrocarbons (PAHs) in street dust of Abadan, Iran. Catena, 177, 246–259.

Ghorbani, M. R., Ghanavati, N., Babaenejad, T., Nazarpour, A., & Payandeh, K. (2020). Assessment of the potential ecological and human health risks of heavy metals in Ahvaz oil field, Iran. PLoS ONE, 15(11), e0242258.

Hakanson, L. (1980). An ecological risk index for aquatic pollution control: A sedimentological approach. Water Research, 14, 975–1001.

Hansen, B. R., & Davies, S. R. H. (1994). Review of potential technologies for the removal of dissolved components from produced water. Chemical Engineering Research and Design, 72(A2).

Hassan Shetol, M., Moklesur Rahman, M., Sarder, R., Ismail Hossain, M., & Kabir Riday, F. (2019). Present status of Bangladesh gas fields and future development: A review. Journal of Natural Gas Geoscience, 4(6), 347–354.

Hossain, M. S., Khan, M. S. H., Chowdhury, K. R., Abdullah. R. (2019). Synthesis of the tectonic and structural elements of the bengal basin and its surroundings [Internet]. Springer Geology. Springer International Publishing, 135–218. http://dx.doi.org/10.1007/978-3-319-99341-6_6.

Hu, L., Yu, J., Luo, H., Wang, H., Xu, P., & Zhang, Y. (2020). Simultaneous recovery of ammonium, potassium and magnesium from produced water by struvite precipitation. Chemical Engineering Journal, 382, 123001. https://doi.org/10.1016/j.cej.2019.123001

Igunnu, E. T., & Chen, G. Z. (2014). Produced water treatment technologies. International Journal of Low-Carbon Technologies, 9(3), 157–177. https://doi.org/10.1093/ijlct/cts049

International Atomic Energy Agency. (2004). Radiation and waste safety in the oil and gas industry (Safety Report No. 34). IAEA.

International Energy Agency. (2019). Global Energy Demand Grew by 2.1% in 2017, and Carbon Emissions Rose for the First Time since 2014. Accessed 21 January, 2019. https://www.iea.org/news/global-energy-demand-grew-by-21-in-2017-and-carbon-emissions-rose-for-the-first-time-since-2014

Jonkers, G., Hartog, F. A., Knaepen, W. A. I., Lancee, P. F. J. (1997). Characterization of NORM in the oil and gas production (E&P) industry. In proceedings of the International Symposium on Radiological Problems with Natural Radioactivity in the Non-nuclear Industry. Amsterdam, www.researchgate.net/publication/288883050_Characterization_of_NORM_in_the_oil_and_gas_production_EP_industry

Johnson, B. M., Kanagy, L. E., Rodgers, J. H., & Castle, J. W. (2008). Feasibility of a pilot-scale hybrid constructed wetland treatment system for simulated natural gas storage produced waters. Environmental Geosciences, 15(3), 91–104.

Johnson, S. Y., & Nur Alam, A. M. (1991). Sedimentation and tectonics of the Sylhet trough, Bangladesh. Geological Society of America Bulletin, 103(11), 1513–1527.

Jolly, Y. N., Rakib, R. J., Kumar, R., Reza, A., Islam, M. T., Rabby, A., et al. (2023). Deciphering the source of heavy metals in industrially affected river sediment of Shitalakshya River, Bangladesh, and potential ecological and health implications. Journal of Hazardous Materials Advances, 10, 100268. https://doi.org/10.1016/j.hazadv.2023.100268

Jolly, Y. N., Rakib, M. R. J., Sakib, M. S., Shahadat, M. A., et al. (2022). Impact of Industrially Affected Soil on Humans: A Soil-Human and Soil-Plant-Human Exposure Assessmen. Toxics, 10, 347. https://doi.org/10.3390/toxics10070347

Karbassi, A. R., Tajziehchi, S., & Afshar, S. (2015). An investigation on heavy metals in soils around oil field areas. Global Journal of Environmental Science and Management, 1(4), 275–282. https://doi.org/10.7508/gjesm.2015.04.002

Khan, R., Das, S., Kabir, S., Habib, M. A., et al. (2019). Evaluation of the elemental distribution in soil samples collected from shipbreaking areas and an adjacent island. Journal of Environmental Chemical Engineering, 7(3), 103189. https://doi.org/10.1016/j.jece.2019.103189

Khan, R., Parvez, M. S., Jolly, Y. N., Haydar, M. A., Alam, M. F., Khatun, M. A., Sarker, M. M. R., & Habib, M. A. (2019). Elemental abundances, natural radioactivity and physicochemical records of a southern part of Bangladesh: Implication for assessing the environmental geochemistry. Environmental Nanotechnology, Monitoring & Management, 12, 100225. https://doi.org/10.1016/j.enmm.2019.100225

Li, F., Qiu, Z., Zhang, J., et al. (2017). Spatial distribution and fuzzy health risk assessment of trace elements in surface water from Honghu Lake. International Journal of Environmental Research and Public Health, 14, 1011.

Liu, X., Song, Q., Tang, Y., et al. (2013). Human health risk assessment of heavy metals in soil–vegetable systems: A multi-medium analysis. Science of the Total Environment, 463, 530–540. https://doi.org/10.1016/j.scitotenv.2013.06.064

Ma, X., Zuo, H., Tian, M., Zhang, L., et al. (2016). Assessment of heavy metals contamination in sediments from three adjacent regions. Environmental Science and Pollution Research, 23, 17238–17247.

Malik, D. S., & Maurya, P. K. (2014). Heavy metal concentration in water, sediment, and tissues of fish species (Heteropneustis fossilis and Puntius ticto) from Kali River, India. Toxicological & Environmental Chemistry, 96(8), 1195–1206.

Mayor, P., Soliño, L., Cartró-Sabaté, M., & Orta-Martínez, M. (2024). Impact of hydrocarbon extraction on heavy metal concentrations in lowland paca (Cuniculus paca) from the Peruvian Amazon. Science of the Total Environment, 930, 172371. https://doi.org/10.1016/j.scitotenv.2024.172371

Menzie, C. A. (1982). The environmental implications of offshore oil and gas activities. Environmental Science & Technology, 16(8), 454A–472A.

Nazarpour, A., Watts, M. J., Madhani, A., & Elahi, S. (2019). Source, spatial distribution and pollution assessment of Pb, Zn, Cu, and Pb isotopes in urban soils of Ahvaz City, a semi-arid metropolis in southwest Iran. Scientific Reports, 9(1), 1–11. https://doi.org/10.1038/s41598-018-37186-2

Najman, Y., Allen, R., Willett, E. A. F., Carter, A., Barfod, D., Garzanti, E., et al. (2012). The record of Himalayan erosion preserved in the sedimentary rocks of the Hatia Trough of the Bengal Basin and the Chittagong Hill Tracts, Bangladesh. Basin Res [Internet]. Oct 20, 24(5), 499–519. https://onlinelibrary.wiley.com/doi/10.1111/j.1365-2117.2011.00540.x

Neff, J. M. (2002). Bioaccumulation in marine organisms: Effects of contaminants from oil well produced water. Elsevier.

Neff, J., Lee, K., & DeBlois, E. M. (2011). Produced water: Overview of composition, fates, and effects. In Produced Water (pp. 3–54). Springer.

NRPA. (2004). Natural radioactivity in produced water from the norwegian oil and gas industry in 2003. Norwegian Radiation Protection Authority Report. 005, 2.

Ogunkunle, C. O., & Fatoba, P. O. (2013). Pollution loads and ecological risk assessment of soil heavy metals around a mega cement factory in southwest Nigeria. Polish Journal of Environmental Studies, 22(2), 487–493.

Orosun, M. M. (2021). Assessment of arsenic and its associated health risks due to mining activities in parts of north-central Nigeria: Probabilistic approach using Monte Carlo simulation. Journal of Hazardous Materials, 412, 125262.

Qing, X., Yutong, Z., & Shenggao, L. (2015). Assessment of heavy metal pollution and human health risk in urban soils of a steel industrial city (Anshan), Liaoning, northeast China. Ecotoxicology and Environmental Safety, 120, 377–385. https://doi.org/10.1016/j.ecoenv.2015.06.019

Rahman, M. J. J., Xiao, W., McCann, T., & Songjian, A. (2017). Provenance of the Neogene Surma Group from the Chittagong Tripura Fold Belt, southeast Bengal Basin, Bangladesh: Constraints from whole-rock geochemistry and detrital zircon U–Pb ages. Journal of Asian Earth Sciences, 148, 277–293.

Rahman, M. J. J., Xiao, W., Hossain, M. S., Yeasmin, R., Sayem, A. S. M., & Ao, S. (2020). Geochemistry and detrital zircon U–Pb dating of Pliocene–Pleistocene sandstones of the Chittagong Tripura Fold Belt (Bangladesh): Implications for provenance. Gondwana Research, 78, 278–290.

Rahman, M. S., Khan, M. D. H., Jolly, Y. N., Kabir, J., et al. (2019). Assessing risk to human health for heavy metal contamination through street dust in the Southeast Asian megacity: Dhaka, Bangladesh. Science of the Total Environment, 660, 1610–1622.

Rahman, M. S., Rahman, M., Jolly, Y. N., Hossain, M. K., Semme, S. A., Paraye, B. A., Arai, T., Yu, J., & Hossain, M. B. (2024). Heavy metals in afforested mangrove sediment from the world's largest delta: Distributional mapping, contamination status, risk assessment and source tracing. Marine Pollution Bulletin, 203, 116429. https://doi.org/10.1016/j.marpolbul.2024.116429

Rakib, R. J., Hossain, M. B., Jolly, Y. N., Akther, S., Islam, S. (2021). EDXRF Detection of Trace Elements in Salt Marsh Sediment of Bangladesh and Probabilistic Ecological Risk Assessment, Soil and Sediment Contamination: An International Journal, DOI:10.1080/15320383.2021.1923644

Rashid, M. B., Sheik, M. R., Haque, A. J. M. E., Siddique, M. A. B., Habib, M. A., & Patwary, M. A. A. (2023). Salinity-induced change in green vegetation and land use patterns using remote sensing, NDVI, and GIS techniques: A case study on the southwestern coast of Bangladesh. Case Studies in Chemical and Environmental Engineering, 7, 100314.

Rashid, M. B., Habib, M. A., Mahmud, A., Ahsan, M. K., Khasru, M. H., Hossain, M. A., et al. (2023). Tectonic setting, provenance, depositional, and paleo-climatic conditions of the late quaternary subcrop sediments of the southeastern coastal region of the Bengal basin. Heliyon. Jan, 9(1), e12998.

Ray, J. P., Rainer, F., Engelhardt. (1992). Produced water: technological/environmental issues and solutions. Environ Sci Res, 46, 1–5.

Reátegui-Zirena, E. G., Stewart, P. M., Whatley, A., Chu-Koo, F., Sotero-Solis, V. E., Merino-Zegarra, C., et al. (2014). Polycyclic aromatic hydrocarbon concentrations, mutagenicity, and Microtox acute toxicity testing of Peruvian crude oil and oil-contaminated water and sediment. Environ Monit Assess [Internet]. Apr, 30, 186(4), 2171–84. http://link.springer.com/10.1007/s10661-013-3527-2.

Roser, B. P., & Korsch, R. J. (1988). Provenance signatures of sandstone–mudstone suites determined using discriminant function analysis of major-element data. Chemical Geology, 67, 119–139.

Rubio, B., Nombela, M. A., Vilas, F. (2000). Geochemistry of major and trace elements in sediments of the Ria de Vigo (NW Spain): an assessment of metal pollution. Marine Pollution Bulletin, 40(11), 968–980.

Rudnick, R. L., Gao, S. (2014). Composition of the Continental Crust. Treatise on Geochemistry, 2nd ed.). Chapter 4, pp, 1–64.

Saleh, H. M., Eskander, S. B., Mahmoud, H. H., Abdelaal, S.A. (2022). Study on rare earth elements, heavy metals and organic contents in the soil of oil exploration site at Matruh Governorate, Egypt. Results Geophys Sci, Mar 1, 9:100039.

Salem, F., Thiemann, T. (2022). Produced Water from Oil and Gas Exploration—Problems, Solutions and Opportunities. J Water Resour Prot [Internet], 14(02), 142–85. https://www.scirp.org/journal/doi.aspx?doi=10.4236/jwarp.2022.142009

Spacil, M, M., Rodgers, J. H., Castle, J. W., Chao, W. Y. (2011). Performance of a pilot-scale constructed wetland treatment system for selenium, arsenic, and low-molecular-weight organics in simulated fresh produced water. Environ Geosci [Internet], Sep, 18(3), 145–56. http://search.datapages.com/data/doi/10.1306/eg.01281110020.

Tamim, U., Khan, R., Jolly, Y. N., Fatema. K., et al. (2016). Elemental distribution of metals in urban river sediments near an industrial effluent source. Chemospher, 155, 509–518.

Tao, X. Q., Shen, D. S., Shentu, J. L., et al. (2015). Bioaccessibility and health risk of heavy metals in ash from the incineration of different e-waste residues. Environmental Science and Pollution Research, 22(5), 69-3558, https://doi.org/10.1007/s11356-014-3562-8 PMID: 25249049

Uddin, A., & Lundberg, N. (1998). Unroofing history of the eastern Himalaya and the Indo-Burman Ranges: Heavy mineral study of Cenozoic sediments from the Bengal Basin, Bangladesh. Journal of Sedimentary Research, 68(3), 465–472.

US EPA. (2001). U.S. Environmental Protection Agency. Supplemental Guidance for Developing Soil Screening Levels for Superfund Sites; Office of Solid Waste and Emergency Response (OSWER). Washington, DC, USA.

Utvik, T. I. R. (1999). Chemical Characterization of Produced Water from Four Offshore Oil Production Platforms in the North Sea. Chemosphere, 39, 2593-2606. 
https://doi.org/10.1016/S0045-6535(99)00171-X

Vaikosen, E. N., Ebeshi, B. U., Airhihen, B. (2014). Bioaccumulation of Heavy Metals and Hydrocarbons in Hemichromis Fasciatus Exposed to Surface Water in Borrow Pits Located Within Onshore Oil Exploration and Production Area. Environ Pollut [Internet], Jun, 28, 3(3). http://ccsenet.org/journal/index.php/ep/article/view/38236. doi:10.5539/ep.v3n3p38

Wei, X., Gao, B., Wang, P., et al. (2015). Pollution characteristics and health risk assessment of heavy metals in street dusts from different functional areas in Beijing, China. Ecotoxicology and environmental safety, 112, 92-186. https://doi.org/10.1016/j.ecoenv.2014.11.005 PMID: 25463870.

Wilk, A., Kalisińska, E., Kosik-Bogacka, D. I., Romanowski, M., Różański, J., & Ciechanowski, K. (2017). Cadmium, lead and mercury concentrations in pathologically altered human kidneys. Environmental Geochemistry and Health, 39(4), 889–899.

Xiao, F. (2021). Characterization and treatment of Bakken oilfield produced water as a potential source of value-added elements. Sci Total Environ [Internet], May, 20, 770,145283. https://linkinghub.elsevier.com/retrieve/pii/S0048969721003491.

Yang, H. J., Lee, C. Y., Chiang, Y. J., et al. (2016). Distribution and hosts of arsenic in a sediment core from the Chianan Plain in SW Taiwan: Implications on arsenic primary source and release mechanisms. Sci Total Environ [Internet], 569–570, 212–22. http://dx.doi.org/10.1016/j.scitotenv.2016.06.122.

Yang, L., Xiao, W., Rahman, M. J. J., Windley, B. F., Schulmann. K., Ao, S., et al. (2020). Indo-Burma passive amalgamation along the Kaladan Fault: Insights from zircon provenance in the Chittagong-Tripura Fold Belt (Bangladesh). Bull Geol Soc Am, 132(9–10), 1953–68.

Zhao, S., Feng, C., Yang, Y., Niu, J., Shen, Z. (2012). Risk assessment of sedimentary
elements in the Yangtze Estuary: new evidence of the relationships between two
typical index methods. J. Hazard. Mater, 241–242, 164–172.

Etim, E. E., Samuel, H. S., Undie, D. A., Akinpelu, O. O., Ibekwe, F. A., & Onotu, O. P. (2026). Environmental management practices in the oil and gas industry moving beyond conventional perceptions of harm. Discover Environment, 4(1). https://doi.org/10.1007/s44274-025-00505-2

Alamu, R., & Dada, O. J. (2025). Sedimentary Basins and Hydrocarbon Accumulation.

Oghenevovwero, E. E., & Gregory, O. A. (2018). Radiological Hazards Assessment of Produced Water from Some Oil and Gas Flow Stations in Delta State, Nigeria. Journal of Scientific Research and Reports, 19(6), 1–10. https://doi.org/10.9734/JSRR/2018/41478

Stepanova, A. Y., Gladkov, E. A., Osipova, E. S., Gladkova, O. V., & Tereshonok, D. V. (2022). Bioremediation of Soil from Petroleum Contamination. Processes, 10(6), 1224. https://doi.org/10.3390/pr10061224

Ilinykh, G., Fellner, J., Sliusar, N., & Korotaev, V. (2023). A life cycle assessment of drilling waste management: a case study of oil and gas condensate field in the north of western Siberia, Russia. Sustainable Environment Research, 33(1), 9-. https://doi.org/10.1186/s42834-023-00171-0

‌

‌

Appendix

Table A1. Enrichment factor values of Shahbazpur gas field environmental soil and sediment samples along with gas well core samples

| Sample ID | K | Ca | Mn | Fe | Ni | Zn | Sr | Y | Zr |  | Ti | As | Pb |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EB-1 | 0.391 | 1.297 | 0.701 | 1.000 | 1.497 | 1.737 | 0.492 | 1.871 | 1.552 |  | 1.200 | 6.898 | 5.114 |
| EB-2.1 | 0.356 | 1.272 | 0.825 | 1.000 | 1.404 | 1.041 | 0.329 | 2.153 | 1.589 |  | 1.307 | 5.800 | 4.465 |
| EB-2.2 | 0.368 | 1.734 | 0.794 | 1.000 | 1.973 | 1.634 | 0.449 | 2.146 | 1.571 |  | 0.955 | 6.991 | 5.382 |
| EB-3.1 | 0.152 | 0.659 | 0.317 | 1.000 | 0.477 | 0.787 | 0.334 | 1.193 | 1.092 |  | 1.273 | 2.079 | 2.582 |
| EB-3.2 | 0.202 | 0.715 | 0.538 | 1.000 | 0.882 | 1.598 | 0.353 | 1.634 | 1.064 |  | 0.584 | 3.169 | 2.893 |
| EB-4.1 | 0.299 | 1.199 | 0.642 | 1.000 | 1.242 | 1.498 | 0.316 | 1.479 | 1.273 |  | 1.015 | 5.721 | 4.687 |
| EB-4.2 | 0.095 | 0.805 | 0.653 | 1.000 | 0.558 | 1.250 | 0.414 | 2.003 | 1.199 |  | 1.074 | 1.829 | 1.701 |
| EB-5 | 0.401 | 1.269 | 0.649 | 1.000 | 1.917 | 1.058 | 0.374 | 1.337 | 1.095 |  | 1.214 | 5.897 | 4.540 |
| EB-6 | 1.293 | 5.035 | 2.801 | 1.000 | 4.572 | 4.002 | 3.140 | 2.974 | 1.498 |  | 23.305 | 17.116 | 12.483 |
| EB-7 | 0.508 | 2.131 | 1.085 | 1.000 | 2.160 | 1.969 | 0.665 | 2.224 | 1.832 |  | 1.778 | 12.447 | 9.078 |
| EB-8 | 0.410 | 0.502 | 0.629 | 1.000 | 0.398 | 1.270 | 0.282 | 1.363 | 1.057 |  | 1.015 | 1.562 | 1.028 |
| CB-1 | 0.191 | 0.748 | 0.392 | 1.000 | 0.678 | 1.589 | 0.298 | 1.235 | 0.981 |  | 0.911 | 2.097 | 2.390 |
| CB-2 | 0.326 | 1.051 | 0.738 | 1.000 | 1.528 | 1.327 | 0.347 | 1.545 | 1.302 |  | 1.151 | 5.810 | 4.238 |
| CB-3 | 0.291 | 0.905 | 0.474 | 1.000 | 0.971 | 1.269 | 0.285 | 1.511 | 0.937 |  | 0.822 | 4.310 | 3.195 |

Table A2. Geo-accumulation index values of Shahbazpur gas field environmental soil and sediment samples along with gas well core samples

| Sample ID | K | Ca | Ti | Mn | Fe | Ni | Zn | As | Sr | Y | Zr | Pb |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EB-1 | -1.936 | -0.207 | -0.320 | -1.095 | -0.582 | 0.000 | 0.214 | 2.204 | -1.605 | 0.321 | 0.052 | 1.772 |
| EB-2.1 | -1.880 | -0.044 | -0.005 | -0.668 | -0.391 | 0.098 | -0.333 | 2.145 | -1.994 | 0.715 | 0.277 | 1.768 |
| EB-2.2 | -1.941 | 0.294 | -0.566 | -0.832 | -0.500 | 0.481 | 0.209 | 2.306 | -1.656 | 0.602 | 0.152 | 1.929 |
| EB-3.1 | -3.026 | -0.907 | 0.044 | -1.960 | -0.304 | -1.373 | -0.650 | 0.751 | -1.886 | -0.049 | -0.178 | 1.064 |
| EB-3.2 | -2.620 | -0.796 | -1.088 | -1.208 | -0.312 | -0.493 | 0.364 | 1.352 | -1.813 | 0.396 | -0.223 | 1.220 |
| EB-4.1 | -2.056 | -0.050 | -0.291 | -0.952 | -0.312 | 0.000 | 0.270 | 2.204 | -1.972 | 0.252 | 0.036 | 1.916 |
| EB-4.2 | -3.693 | -0.616 | -0.201 | -0.918 | -0.304 | -1.145 | 0.019 | 0.568 | -1.575 | 0.699 | -0.042 | 0.463 |
| EB-5 | -1.730 | -0.067 | -0.131 | -1.034 | -0.411 | 0.528 | -0.329 | 2.149 | -1.831 | 0.009 | -0.280 | 1.772 |
| EB-6 | -1.578 | 0.384 | 2.595 | -0.462 | -1.948 | 0.245 | 0.053 | 2.149 | -0.297 | -0.376 | -1.365 | 1.694 |
| EB-7 | -2.005 | 0.062 | -0.199 | -0.911 | -1.029 | 0.082 | -0.052 | 2.608 | -1.618 | 0.124 | -0.156 | 2.153 |
| EB-8 | -1.482 | -1.189 | -0.172 | -0.864 | -0.194 | -1.524 | 0.150 | 0.449 | -2.020 | 0.252 | -0.114 | -0.154 |
| CB-1 | -1.600 | 0.368 | 0.653 | -0.564 | 0.787 | 0.227 | 1.455 | 1.855 | -0.960 | 1.091 | 0.758 | 2.043 |
| CB-2 | -2.009 | -0.322 | -0.191 | -0.831 | -0.394 | 0.218 | 0.014 | 2.145 | -1.919 | 0.234 | -0.013 | 1.690 |
| CB-3 | -1.684 | -0.044 | -0.184 | -0.979 | 0.099 | 0.057 | 0.442 | 2.207 | -1.711 | 0.695 | 0.006 | 1.775 |

Table A3. Contamination factor values of Shahbazpur gas field environmental soil and sediment samples along with gas well core samples

| Sample ID | K | Ca | Mn | Fe | Ni | Zn | Sr | Y | Zr |  | Ti | As | Pb |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EB-1 | 0.392 | 1.300 | 0.702 | 1.002 | 1.500 | 1.740 | 0.493 | 1.874 | 1.555 |  | 1.202 | 6.910 | 5.123 |
| EB-2.1 | 0.407 | 1.455 | 0.944 | 1.144 | 1.606 | 1.190 | 0.377 | 2.463 | 1.818 |  | 1.495 | 6.635 | 5.108 |
| EB-2.2 | 0.391 | 1.840 | 0.843 | 1.061 | 2.093 | 1.733 | 0.476 | 2.277 | 1.666 |  | 1.014 | 7.418 | 5.711 |
| EB-3.1 | 0.184 | 0.800 | 0.386 | 1.215 | 0.579 | 0.956 | 0.406 | 1.450 | 1.326 |  | 1.546 | 2.525 | 3.136 |
| EB-3.2 | 0.244 | 0.864 | 0.649 | 1.208 | 1.066 | 1.930 | 0.427 | 1.974 | 1.285 |  | 0.705 | 3.828 | 3.495 |
| EB-4.1 | 0.361 | 1.449 | 0.775 | 1.208 | 1.500 | 1.809 | 0.382 | 1.787 | 1.538 |  | 1.226 | 6.910 | 5.662 |
| EB-4.2 | 0.116 | 0.979 | 0.794 | 1.215 | 0.678 | 1.520 | 0.503 | 2.435 | 1.457 |  | 1.305 | 2.223 | 2.067 |
| EB-5 | 0.452 | 1.432 | 0.733 | 1.128 | 2.163 | 1.194 | 0.421 | 1.509 | 1.235 |  | 1.370 | 6.654 | 5.123 |
| EB-6 | 0.503 | 1.958 | 1.089 | 0.389 | 1.777 | 1.556 | 1.221 | 1.156 | 0.582 |  | 9.061 | 6.654 | 4.853 |
| EB-7 | 0.374 | 1.566 | 0.798 | 0.735 | 1.587 | 1.447 | 0.489 | 1.635 | 1.346 |  | 1.307 | 9.148 | 6.672 |
| EB-8 | 0.537 | 0.658 | 0.824 | 1.311 | 0.522 | 1.664 | 0.370 | 1.787 | 1.386 |  | 1.331 | 2.048 | 1.348 |
| CB-1 | 0.495 | 1.936 | 1.015 | 2.588 | 1.755 | 4.112 | 0.771 | 3.196 | 2.537 |  | 2.358 | 5.425 | 6.183 |
| CB-2 | 0.373 | 1.200 | 0.843 | 1.142 | 1.744 | 1.515 | 0.397 | 1.764 | 1.487 |  | 1.314 | 6.635 | 4.839 |
| CB-3 | 0.467 | 1.454 | 0.761 | 1.606 | 1.560 | 2.038 | 0.458 | 2.428 | 1.506 |  | 1.320 | 6.924 | 5.133 |