Urbanization and its impact on Land Surface Temperature: A case study in Coastal Odisha, India

Abstract

The East and South Eastern Coastal Plain Zone (ESECPZ) of Odisha, situated along the eastern coast of India, present a mosaic of urban and rural regions. The present study aims to estimate the land use land cover change and extent of urbanization in ESECPZ over the span of two decades, from 2000 to 2020. Multi-year Landsat data underwent supervised maximum likelihood classification to estimate LULC change. The ESECPZ of Odisha underwent substantial transformations during the study period. Forest cover decreased from 33.95% to 18.98%, agricultural land decreased from 24.98% to 21.84%, as against the settlements which surged from 21.52% to 39.67%. The maximum, minimum and average Land Surface Temperature (LST) showed steady increase during the period of study. About 5°C increase in the minimum LST range and 3°C increase in maximum LST were recorded. The increase in LST was attributed to the expansion of settlement areas and reduction in the vegetation cover due to population growth and urbanization. Normalized Difference Vegetation Index (NDVI) and Normalized Difference Built-up Index (NDBI) showed gradual decreasing and increasing trends, respectively. Strong positive correlation was found between LST and NDBI while a strong negative correlation was found between LST and NDVI during all the years under study. This research provides valuable insights into the often-overlooked impacts of urbanization on (LST) and vegetation cover; the findings offer a positive outlook for the future, providing a foundation for forward-thinking initiatives aimed at addressing these environmental challenges.

Keywords – Landsat, LST, NDBI, NDVI, Urbanization

- Introduction

Rapid urbanization and industrialization are two interconnected phenomena that are posing significant challenges to the planet Earth (Bai et al., 2017). The rapid influx of people into urban areas, driven by factors such as migration and birth rates, cause urban areas to grow at an alarming rate (Virtudes et al., 2017). As a result, population growth coupled with the increasing industrialization and production demands, pose immense pressure on the Earth's resources and ecosystems (Motesharrei et al., 2016), leading to environmental degradation, climate change, and non-renewable natural resource scarcity. It has been noted that the amount of land covered by urbanized areas will increase twice over the course of the next 40 years and most people on the planet would be impacted by anthropogenic climate change in urban contexts (Zhao et al., 2019). Therefore, the importance of local urban governments in addressing climate change should be emphasized (Khosla and Bhardwaj, 2019; Parapurath et al., 2025a).

The degree of temperature variation between urban and rural areas tends to increase with higher levels of urbanization (Tam et al., 2015; Chapman et al., 2017; Chen et al., 2017; Peng et al., 2021). Notably, natural cooling effects are eliminated when natural land cover is replaced with infrastructure, concrete, and industrial activity (Mohajerani et al., 2017). Moreover, one of the harmful effects of urbanization on Land Surface Temperature (LST) is the creation of heat islands (Tam et al., 2015; Gowtham and Dhanapriya, 2025). Heat islands are localized areas within urban environments that experience significantly higher temperatures compared to the surrounding rural areas (Mardiatmoko & Hatulesila, 2020). These heat islands are caused by the high concentration of buildings, roads, and concrete surfaces in urban areas, which absorb and retain more heat from the sun compared to natural landscapes (Irfeey et al., 2023). The high LST can make urban areas extremely uncomfortable and even dangerous to live in, especially during heatwaves (Macintyre et al., 2018; Wei et al., 2024). Additionally, the elevated LST contribute to increased energy consumption for cooling purposes, leading to higher greenhouse gas emissions (Van Vuuren et al., 2017). As urban areas become more developed, they contribute to an increase in the air temperature through the release of heat from buildings, vehicles and industrial activities (Santos et al., 2020). Argüeso et al. (2014) predict that minimum temperatures will rise more rapidly in recently developed areas, with these changes occurring throughout the year. Furthermore, as the global climate changes, cities will be increasingly vulnerable to extreme weather events such as heatwaves, storms, and flooding (Chen et al., 2014; Wang et al., 2015). The faster rate of change in climate variables combined with the occurrences of extreme weather events (Senapati et al., 2024; Parapurath et al., 2025b; Subba Rao et al., 2025; Meena et al., 2026) have significant impact on the land use classes.

Throughout the twentieth century, the expansion of urbanization globally has led to a decline in tree cover within urban areas, largely driven by the demand for land development (Güneralp et al., 2020). The transformation of agricultural lands into urban areas has also disrupted the local hydrological cycle, leading to reduced groundwater recharge and increased surface runoff (Nayak and Nandimandalam, 2023). However, satellite-based remote sensing has been proved to be a valuable tool for assessing and monitoring urbanization in India (Radočaj et al., 2020). It allows decision makers to precisely monitor the changes in land cover and provides crucial information for public policy planning and the management of earth resources, enabling decision makers to address issues like changes of land surface temperature, soil moisture, surface water resources, extent of agriculture etc. (Swain et al., 2017; Nanda et al., 2023; Ghosh et al., 2024). In order to monitor the extent of urbanization over time, researchers successfully harnessed the potential of high-resolution satellite datasets viz., Landsat, MODIS (Moderate Resolution Imaging Spectroradiometer), and Sentinel (Sidiqui et al., 2016; Bonafoni and Keeratikasikorn, 2018). While numerous studies have gauged the warmth of urban areas by measuring air temperature through ground-based observation stations, but monitoring LST provides a more comprehensive perspective on the thermal dynamics of these regions because among the indicators, LST is a valuable indicator that offers crucial insights into the physical characteristics of the Earth's surface and its influence on climate, which in turn affects various environmental processes (Wu et al., 2019). Additionally, several researchers have computed different satellite derived indices such as Enhanced Vegetation Index (EVI), Normalized Difference Built-up Index (NDBI), Normalized Difference Vegetation Index (NDVI) etc., to estimate the extent of urbanization, and vegetation cover (Karanam and Neela, 2017; Kumari et al., 2018; Bouhennache et al., 2019; Parapurath et al., 2020). So, incorporating Remote Sensing (RS) and Geographic Information System (GIS) enables comprehensive mapping by integrating diverse geospatial data for enhanced analysis (Parapurath et al., 2025c; Prakash et al., 2025).

In the coming decades, India will experience the greatest urban shift on record world-wide because there are currently more than 50 urban agglomerations in the nation with a million citizens. Further, it is anticipated that India's urban population would double to over 800 million people (Chakrabarti, 2001). The increasing urbanization in India has led to significant changes in land use patterns. These changes coupled with climate change impacts have created a complex and challenging situation for the country (Roy et al., 2022; Athul et al., 2025). Being no exception, Odisha, the state on the east coast of India, is experiencing urbanization occurring at a steady pace (Zhao et al., 2017; Lohani, 2022). As a result of urbanization, there have been significant changes in the climate of Odisha including extreme high temperature, heat waves, altered precipitation patterns, and the formation of urban heat islands (Gogoi et al., 2019).

Therefore, the present study aims to monitor the spatial and temporal changes in land use & land cover patterns for understanding the urbanization extent of East and South Eastern Coastal Plain Zone of Odisha from 2000-2020 and to assess the impact of urbanization on land surface temperature using geospatial technology.

- Materials and methods

2.1 Study area

The East and South Eastern Coastal Plain Zone (ESECPZ) of Odisha (Fig. 1) is geographically distributed along the 290 km long sea coast and stretches to 100–120 km inland from the sea (Behera et al., 2023). The present study was conducted in the five districts of ESECP viz. Puri, Kendrapara, Jagatsinghpur, Khordha, and Nayagarh among which three districts namely Puri, Kendrapara and Jagatsinghpur are located along the coast of the Bay of Bengal. The demographic profile of the districts is given in table 1. The zone is characterized by a hot, humid tropical climate with predominantly lateritic, alluvial and saline soils. The study area receives an average annual rainfall of 1450 mm about 80% of which falls during the monsoon season. The average summertime temperature is 39°C, while the average winter low temperature is 11.5°C (Sahu et al., 2021; Dugal et al., 2022).

Fig. 1 Geographical location of the East and South Eastern Coastal Plain Zone (ESECPZ) of Odisha

Table 1 Demographic profile of the ESECPZ of Odisha

| District | Geographic area (km2) | Population density per sq. km |  |  |
| --- | --- | --- | --- | --- |
|  |  | 1991 | 2001 | 2011 |
| Puri | 3,479 | 375.2 | 431.9 | 488.3 |
| Kendrapara | 2,644 | 434.8 | 492.4 | 544.8 |
| Jagatsinghapur | 1,668 | 560.0 | 634.2 | 681.6 |
| Khordha | 2,813 | 534.0 | 667.4 | 800.5 |
| Nayagarh | 3,890 | 201.2 | 222.2 | 247.5 |

2.2 Data

Multi-year Landsat data were used in the present study for land use and land cover monitoring. In order to monitor the decadal changes in LULC and thereby urban extent, Landsat 7 ETM+, Landsat 5 MSS and Landsat 8 OLI and TRIS data were collected for the year 2000, 2010 and 2020, respectively from the open-data archive of the United States Geological Survey (USGS) via www.earthexplorer.usgs.gov. Imageries taken in the visible, near infra-red and shortwave infra-red channels were used for land use classification while the imageries taken in the thermal infra-red channel were used for estimating Land Surface Temperature. The study area was covered by two Landsat tiles (139/46 and 140/46). So, two data were downloaded for each year. The details of the data used in the study have been given in table 2. Google Earth Pro imageries were used for generating the Ground truth points for training the classification and testing the accuracy.

Table 2 Details of the data used in the study

| Year | Acquisition Date | Path/ Row | Satellite and Sensors | Bands used | Resolution |
| --- | --- | --- | --- | --- | --- |
| 2000 | 17/12/2000 10/12/2000 | 139/46 140/46 | Landsat 7 ETM+ (Enhanced Thematic Mapper) | Band 1: Blue (0.45-0.52 μm) Band 2: Green (0.52-0.60 μm) Band 3: Red (0.63-0.69 μm) Band 4: Near-Infrared (0.76-0.90 μm) Band 5: Near-Infrared (1.55-1.75 μm) Band 6: Thermal (10.40-12.50 μm) Band 7: Mid-Infrared (2.08-2.35 μm) | 30 m 30 m 30 m 30 m 30 m 120 m (MSS) 60 m (ETM+) |
| 2010 | 25/04/2010 18/04/2010 | 139/46 140/46 | Landsat 5 MSS (Multispectral Scanner) |  |  |
| 2020 | 28/03/2020 03/03/2020 | 139/46 140/46 | Landsat 8 OLI (Operational Land Imager) and TIRS (Thermal Infrared Sensor) | Band 2: Visible (0.450-0.51 μm) Band 3: Visible (0.53-0.59 μm) Band 4: Red (0.64-0.67 μm) Band 5: Near-Infrared (0.85-0.88 μm) Band 6: SWIR 1 (1.57-1.65 μm) Band 7: SWIR 2 (2.11-2.29 μm) TIRS Band 10: TIRS 1 (10.6-11.19 μm) Band 11: TIRS 2 (11.5-12.51 μm) | 30 m 30 m 30 m 30 m 30 m 30 m 100m 100 m |

2.3 Image processing and classification

The level-1 satellite data was pre-processed for radiometric calibration and atmospheric correction in Quantum GIS (QGIS) 3.16.8 software. Semi-automatic Classification Plugin (SCP) was applied in radiometric calibration. DOS 1 (Dark Object Subtraction) algorithm was used for atmospheric correction in the present study. Prominently, two tiles viz. 139/ 46 and 140/46 were mosaicked for each image date. The subset image underwent supervised maximum likelihood classification, using the training data from Google Earth Pro. The sample points of the training as well as testing datasets were taken in a stratified way and validated using different bands combination (Khan and Das, 2022) as given in the table 3. The study area was classified into five LULC classes namely Agriculture, Barren land and Rocks, Forest, Settlements and Water bodies (Table 4) using ArcGIS 10.8 software.

Table 3 Landsat band combination applied in the present study

| Features | Band combination for Landsat 5 and 7 | Band combination for Landsat 8 |
| --- | --- | --- |
| Agriculture | 7, 4, 2 | 6, 5, 2 |
| Forest | 4, 5, 3 | 5, 4, 3 |
| Barren land and Rocks | 3, 2, 1 | 5, 6, 4 |
| Settlements | 4, 3, 2 | 7, 6, 4 |
| Water bodies | 3, 2, 1 | 5, 6, 4 |

Table 4 Description of different LULC classes

| Class | Features | Description |
| --- | --- | --- |
| 1 | Agriculture | Cropping lands with crops |
| 2 | Barren land and Rocks | Unused lands and uncultivated lands |
| 3 | Forest | Dense and less dense vegetation |
| 4 | Settlements | Residential and Concrete structures |
| 5 | Water bodies | Ponds, Lakes, Canals and River |

2.4 Accuracy assessment

Classification results were validated using a testing dataset derived from Google Earth Pro images. Classification performance was evaluated using user’s accuracy, producer’s accuracy, and overall accuracy. Furthermore, the kappa coefficient—a measure of agreement between the maximum likelihood classification map and the reference data—was calculated according to Stehman’s formula (1996). Kappa coefficient is less influenced by variations in sample sizes among different classes, making it a more dependable and robust indicator of classification accuracy and the value of the kappa coefficient ranges from 0.1 to 1.0.

K= N i=1knij- i=1kni+njN2- i-1kni-nj

where,

K = number of rows in error matrix; nij = number of observations in row i and column j; ni = total number of observations in row i; nj = total number of observations in column j; N = total number of observations in matrix

Kappa coefficient values can be categorized into five groups (Meer and Mishra, 2020) as given in the following table 5.

Table 5 Categories of Kappa coefficient values

| Kappa coefficient values | Categories |
| --- | --- |
| 0-0.20 | Slight agreement |
| 0.21- 0.40 | Fair agreement |
| 0.41- 0.60 | Moderate agreement |
| 0.61- 0.80 | Substantial agreement |
| 0.81- 1.00 | Almost perfect agreement |

2.5 Estimation of Land Surface Temperature (LST)

The Land Surface Temperature (LST) is calculated using Landsat thermal bands using the following equations adopted by Hakan (2016).

LST= Tb[1+λ*TbC2* ln(ε)]

where, 𝑇𝑏 is at-Satellite Brightness Temperature, λ is wavelength of emitted radiance, 𝐶2 = 1.4388 X 10−2 mK and ε is emissivity (typically 0.95)

Tb= K2(ln(K1*εL+1))

where, K1 is the sensor dependent calibration constant 1 and K2 is the sensor dependent calibration constant 2. ε is emissivity (typically 0.95), and L is the spectral radiance. 𝐶2 is calculated using the following formula

C2= h*cs

where, h is Planck’s constant = 6.626 × 10−34 Js; c is the velocity of light = 2.998 × 108 m/s, and s is the Boltzmann constant = 1.38 × 10−23 J/K.

2.6 Computation of NDVI and NDBI

In order to determine the vegetation condition and urbanization condition, two indices viz. Normalized Difference Vegetation Index (NDVI) and Normalized Difference Built-up Index (NDBI) were computed using the formulae adopted by Alademomi et al. (2022).

NDVI= RNIR-RRRNIR+RR

NDBI= RSWIR-RNIRRSWIR+RNIR

Where, RNIR = Reflectance in near infra-red band; RR= Reflectance in red band and RSWIR= Reflectance in shortwave infra-red band.

2.7 Assessing the relationship of urbanization with LST and air temperature

To determine the impact of urbanization on LST, a correlation analysis was conducted between LST and the estimated area of the settlement class. The influence of surface vegetation cover and built-up area on the surface temperature was examined by the correlation of LST with NDVI and NDBI. The image correlation over the years 2000, 2010, and 2020 was carried out in ArcGIS 10.8 to formally demonstrate the link between LST and vegetation indices.

3 Results

3.1 Decadal LULC change with special reference to urban extent

Maximum likelihood classification of multi-year Landsat data was used to analyze decadal LULC changes in the ESECPZ of Odisha. The thematic LULC maps for the years 2000, 2010 and 2020 have been represented by the figure 2a, 2b and 2c respectively.

The maximum likelihood classification for the year 2000 recorded an overall accuracy of 89.66 % with the kappa coefficient value of 0.87 (Table 6), demonstrating the reliability and acceptability of LULC classification. The results revealed that in the year 2000, forest land exhibited the largest coverage. The estimated area under different LULC classes during the entire study period is given in the Table 7. In 2000, it was estimated that forests covered 4,952.21 sq. km, occupying approximately 34% of the total geographical area. Agricultural land exhibiting the second largest coverage occupied 3644.36 sq. km area which was about 25%. Settlements came third with 3138.67 sq. km area followed by the Barren and rock lands covering 3138.67 sq. km and 1803.44 sq. km respectively. Waterbodies had the smallest coverage with 1047.92 sq. km area which occupied only 7.2% of the total area.

| a. 2000 | b. 2010 | c. 2020 |
| --- | --- | --- |

Fig. 2(a-c) LULC maps of the ESECPZ of Odisha over 2000-2020

Table 6 Accuracies of LULC classification

| Years | Overall Accuracy (%) | Kappa coefficient |
| --- | --- | --- |
| 2000 | 89.66 % | 0.87 |
| 2010 | 88.33 % | 0.85 |
| 2020 | 90.67 % | 0.88 |

Over 2000-2010, the study area experienced a notable decrease in the area under forest. It was estimated that forest area was reduced by about 32.8% during this time period (Table 7). About 23% (3326.25 sq. km) of ESECPZ was classified as forest land in 2010. Agricultural land also followed the declining trend with a reduction of about 3.5% from 2000 to 2010. In 2010, agricultural land covered nearly 24% of the total area. Conversely, settlement areas increased to an estimated 4,712 sq. km, occupying approximately 32%. However, less changes were observed in the area under the Barren and rock lands and waterbodies covering about 14 % (1976.05 sq. km) and 7.2 % (1055.05 sq. km) respectively. Overall, the LULC classification for 2010 also resulted in high accuracy as indicated by the overall accuracy of 88.33 % and a kappa coefficient value of 0.85.

Table 7 Changes in LULC classes (sq.km) of East and South Eastern Coastal Plain Zone of Odisha from 2000 to 2020

| Land use classes | 2000 | 2010 |  | 2020 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Area (sq.km) | Area (sq.km) | Change over 2000 (sq.km) | Area (sq.km) | Change over 2010 (sq.km) | Change over 2000 (sq.km) | % change over 2000 |
| Agriculture | 3644.36 | 3517.25 | -127.11 | 3185.53 | -331.72 | -458.83 | 12.59 |
| Barren Land | 1803.44 | 1976.05 | 172.61 | 1779.93 | -196.12 | -23.51 | 1.30 |
| Forest | 4952.21 | 3326.25 | -1625.96 | 2767.94 | -558.31 | -2184.27 | 44.10 |
| Settlements | 3138.67 | 4712.00 | 1573.33 | 5787.2 | 1075.20 | 2648.53 | 84.38 |
| Water Bodies | 1047.92 | 1055.05 | 7.13 | 1066 | 10.95 | 18.08 | 1.72 |

The LULC classification for the year 2020 revealed further decrease in the areas under forest and agricultural land by 44% and 12.5%, respectively as compared to that in 2000 and 16.78% and 9%, as compared to that in 2010. It was found that forest occupied about 19 % (2767.94 sq. km) while Agricultural land covered about 22% (3185.53 sq. km). The area under the Barren land with 1779.93 sq. km area experienced a slight decrease of 196.12 sq. km when compared to the year 2010. Conversely, there was a substantial increase in the settlement area with 5787.20 sq. km area covering almost 40 % of the total geographical area of the study zone. Notably, there was a marginal increase in the area covered by waterbodies occupying 1066.00 sq. km area. The overall accuracy of the LULC classification was calculated as 90.67% with the kappa coefficient value of 0.88.

The results inferred that the supervised classification demonstrated almost 80 % accurate classification for the Settlements. The highest user’s accuracy of 87.3% was obtained in the year 2020 followed by 80.7% and 78.2 % for the year 2000 and 2010, respectively. On the other hand, the maximum producer’s accuracy was recorded in the year 2000 and calculated as 93.8%. Producer’s accuracies for the year 2010 and 2020 were calculated as 78.2% and 81.4%, respectively. During the study period, a steady rise in the area under Settlements was noticed. In the year 2010, the Settlements area increased by almost 50% from 2000, which implies that an addition of 1573.33 sq. km under the Settlement class. Similarly, in the next decade (over 2010-2020), the area under Settlements increased by about 23%. Overall, the Settlement area observed a highly significant growth of about 84% from 2000-2020.

3.2 Spatiotemporal variation of LST

Landsat thermal bands were used to estimate the Land Surface Temperature (LST) of the study year during 2000, 2010 and 2020 and the results so obtained are been represented in the Figures 3a, 3b, and 3c respectively. In 2000, LST varied from 18.43°C to 42.96°C with an average LST of 30.14°C (Table 8). Although, the average, maximum and minimum LST were found to increase steadily in 2010 and 2020. The average LST increased by 1.68 °C from 2000 to 2010 and 2.97 °C from 2010 to 2020. On the other hand, the maximum and minimum LST increased by 0.8 °C and 2.43°C, respectively from 2000 to 2010. Further increase in the maximum and minimum LST by 2.09 °C and 2.85°C was recorded from 2010 to 2020. Considering the entire study period, the average LST was found to rise by 4.65 °C while the maximum and minimum LST increased by 2.92°C and 5.28°C, respectively.

| a. 2000 | b. 2010 | c. 2020 |
| --- | --- | --- |

Fig. 3(a-c) Spatial variations in the LST of the ESECPZ of Odisha over 2000-2020

Table 8 Temporal changes of LST ranges and its relationship with settlements

| LST | 2000 | 2010 | 2020 | Percentage increase over 2000 (%) | Correlation coefficient between LST and Settlements |
| --- | --- | --- | --- | --- | --- |
| Maximum LST(°C) | 42.96 | 43.79 | 45.88 | 6.79 | 0.950 |
| Minimum LST (°C) | 18.43 | 20.86 | 23.71 | 28.64 | 0.806 |
| Average LST (°C) | 30.14 | 31.82 | 34.79 | 15.42 | 0.955 |

3.3 Spatiotemporal variation of NDVI and NDBI

In order to estimate the spatial variation of vegetation cover over the two decades, NDVI was computed for the three years (2000, 2010 and 2020) and the thematic NDVI maps were represented in the figure 4a, 4b and 4c. The maximum NDVI value was 0.824 in 2000, which subsequently declined to 0.626 in 2010 and 0.583 in 2020, indicating a gradual depletion of vegetation cover.

| a. 2000 | b. 2010 | c. 2020 |
| --- | --- | --- |

Fig. 4(a-c) Spatial pattern of the NDVI of ESECPZ of Odisha over 2000-2020

The spatial pattern of NDBI during the study year was given in the figure 5a, 5b and 5c. The temporal trend of NDBI was found to be just opposite to the trend of NDVI. The results inferred that the highest NDBI values gradually increased from 0.423 in 2000 to 0.638 in 2010 and 0.820 in 2020. This upward trend in NDBI values indicated a progressive expansion of the built-up area in the East and South Eastern Coastal Plain Zone of Odisha over the study period.

| a. 2000 | b. 2010 | c. 2020 |
| --- | --- | --- |

Fig. 5(a-c) Spatial pattern of the NDBI of ESECPZ of Odisha over 2000-2020

3.4 Effect of Urbanization on LST

To ascertain the connection between LST and urbanization, a correlation study was carried out and the results have been given in the Table 8. The maximum LST exhibited a strong positive correlation with the Settlements as indicated by the high value of correlation coefficient (0.95). A similar positive agreement was observed between minimum LST and settlements, with a correlation coefficient of 0.806. However, the correlation coefficient between the average LST and the Settlements was determined as 0.955, indicating a strong positive correlation.

Conversely, a strong inverse relationship was found between the surface vegetation cover and LST as reflected by the negative relation between LST and NDVI in all the three years under consideration (Fig. 6). The values of the coefficient of determination (R2) indicated that the variation in NDVI could explain the changes in LST by 68.2 % in 2000, 63.2 % in 2010 and 61.1 % in 2020. Whereas, a positive relationship was found between LST and NDBI. The results revealed that the NDBI explained maximum variation in LST by 72.2% in 2020 followed by 72.1% and 63 % in 2010 and 2000, respectively.

Fig. 6 Relationship of LST with NDVI and NDBI over the study period

- Discussion

This study successfully utilized multi-temporal Landsat data (MSS, ETM+, and OLI/TIRS) to monitor LULC changes and estimate the extent of urbanization with high accuracy. The legacy of Landsat data, combined with its consistent spatial and temporal resolution, makes it highly applicable for historical LULC change analysis (Dutrieux et al., 2016; Souza et al., 2020). Earlier, researches proved the applicability of multi-year Landsat data for monitoring urbanization with great accuracy (Dou and Chen, 2017). The supervised classification yielded high accuracy, with an overall accuracy exceeding 80% and a kappa coefficient above 0.80, indicating near-perfect agreement (0.81–1.00) between producer and user ratings (Meer & Mishra, 2020; Ghosh et al., 2022).

The current study provided convincing evidence that, during the course of the study period, some land cover types, such as forests and agricultural land, have been gradually disappearing. Conversely, the area set aside for settlements has been growing at a noticeably quicker pace. Forests represent a critical land cover type in Odisha’s East and Southeastern Coastal Plain Zone; however, they have experienced a steady decrease over the past two decades. This originally forested area has seen considerable portioning for urban development. The decline in forest areas due to developmental activities is in line with findings reported by Mishra et al. (2022). The findings suggest that various activities, such as road construction, mining, industrial growth, and agricultural development, are driving land encroachment. The capital of Odisha, Bhubaneswar, is situated in the East and South Eastern Coastal Plain Zone. Due to its tremendous population expansion, Bhubaneswar has seen quick development and construction (Tiwari, 2011; Swain et al., 2016; Swain et al., 2017). Urban expansion has primarily occurred through the reclamation of wetlands and the conversion of agricultural land. Over the past 15 years, the land cover of Bhubaneswar has changed significantly, with a rise in urbanization of 83 percent. In a similar vein, during the course of these 15 years, from 2000 to 2014, there was a significant 89% decline in dense vegetation over the city and 83% decrease in agriculture fields. In the study area, rapid population increase accelerated the rate of urbanization. Puri district (prominent destination for pilgrims), experienced rapid expansion due to rising urbanization. Puri's drawbacks include rising disparities between urban and rural areas and the migration of people from rural to urban areas in search of better economic opportunities. The town's decadal growth rate is 23.9%, which is continuing to grow both horizontally and vertically in the twenty first century (Mishra et al., 2020). More than one third of the East and South Eastern Coastal Plain zone of Odisha is made up of agricultural land, which makes up the majority of the land cover in the study area. But from 2000 to 2020, the area of the agricultural land significantly decreased as a result of strong urbanization pressures, as noted by Gogoi et al. (2019).

The spatiotemporal change in LULC is identified to be one of the major factors responsible for changes in UHI effect over the city (Yan et al., 2016). Numerous previous studies have linked the anthropogenic changes in surface shape brought about by urbanization to notable adjustments in the local surface energy balance, which in turn creates a brand-new, local microclimate. There has been an increase in LST of about 5°C in the low temperature range and 3°C in high temperature range from 2000 to 2020. This rise in temperature can be attributed to increased urbanization. As noted by Liaquat et al. (2018), areas with more developed infrastructure tend to exhibit higher temperatures compared to their surroundings. Urban growth is a primary driver of rising land surface temperatures, fueling a heat-island effect in the metropolitan regions of Odisha's East and Southeastern Coastal Plain Zones. In addition, other elements that may contribute to the increase in land surface temperatures include the spread of arid regions, deforestation, or a decrease in the amount of vegetation cover. Impermeable surfaces such as buildings, roads, and industrial infrastructure absorb incoming shortwave solar radiation and restrict outgoing longwave terrestrial emissions, directly impacting LST (Das et al., 2021). According to Fischer et al. (2012), under global warming, the surrounding rural areas would not experience nearly as much heat stress as the urban areas, and this difference in heat stress would become even more noticeable during heat waves. The ESECPZ’s growing frequency of heat waves is evidence of the detrimental effects of urbanization and LULC change on the region's temperature (Gouda et al., 2017; Guleria and Gupta, 2019; Nageswararao et al., 2020; Neethu et al., 2020; Boyaj et al., 2023).

A progressive decline in NDVI was observed over the study period. Initial low NDVI values in the central region in 2000 transitioned to the eastern and western edges by 2020, indicating increasing vegetation sparse cover. The study area's rising urbanization is the cause of the lowest NDVI values in 2020, which is in line with research showing that deterioration from the conversion of the original vegetated surface to the urbanized impervious surface resulted in lowering of NDVI (Du et al., 2019).  Although agricultural land constitutes over one-third of Odisha’s East and Southeastern Coastal Plain making it the dominant land cover this area experienced a significant decline between 2000 and 2020, driven by intense urbanization pressures (Gogoi et al., 2019).

LST, NDVI and NDBI are closely related (Marzban et al., 2018; Fatemi and Narangifard, 2019; Malik et al., 2019; Alademomi et al., 2022). Through image correlation analysis, it was determined that vegetated regions exhibit a negative correlation with LST, whereas built-up areas demonstrated a strong positive correlation with LST. By using image correlation analysis, it was discovered that built-up areas show a strong positive association with LST, while vegetative zones show a negative correlation (Molnar, 2016). The findings indicated that the study area had a notable rise in land surface temperature between 2010 and 2020 as a result of increasing urbanization. The greatest positive connection was observed between LST and NDBI, whereas lowest negative correlation, was observed between LST and NDVI in 2020.

- Conclusion

The LULC of East and South Eastern Coastal Plain Zone of Odisha had undergone major changes from 2000 to 2020. The forest cover was rapidly diminishing, while settlements were experiencing substantial growth over a 20-year period. The study highlighted significant changes in three primary classes, viz., agriculture, forest land, and settlements. These changes have been driven by the imperative to accommodate the needs of a growing population, resulting in positive growth in settlements and a progressive decrease in both forest coverage and agricultural land.  The expanding urban areas negatively impacted on the LST. Notably, this research has provided valuable insights into the often-overlooked impacts of urbanization on factors such as LST and vegetation cover. By shedding light on these hidden facts, the present study contributes to a better understanding of the complex relationship between urban development and environmental changes. These findings offer a positive perspective for the future and can inform and bolster forward-thinking initiatives and applications aimed at addressing these challenges.

While development is essential for economic growth and housing, mitigating urban heat island effects must be prioritized to achieve sustainability. More investment in green spaces, such as parks, green roofs, and urban forests, to mitigate the urban heat island effect and reduce flooding is required. The construction of energy-efficient and climate-resilient buildings that can withstand extreme weather events should be encouraged. By implementing these recommendations, urbanized areas can become more resilient to the impacts of climate change and create healthier, more sustainable communities for current and future generations.

Furthermore, although satellite imagery has improved significantly, it still lacks the resolution required to capture detailed urban features. This can make it challenging to monitor small-scale urban changes accurately. Tall buildings and vegetation can cast shadows or obscure other features, making it difficult to accurately identify and classify urban land cover. Despite these limitations, satellite remote sensing remains one of the most effective tools for monitoring urbanization at regional and global scales. Integrating satellite data with other sources of information, such as aerial imagery, ground surveys, and census data, can help overcome some of these limitations and provide a more comprehensive understanding of urbanization processes.

Declaration of competing interest

The authors declare no conflicts of interest/competing interests.

Disclaimer (Artificial intelligence)

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

References

Alademomi, A. S., Okolie, C. J., Daramola, O. E., Akinnusi, S. A., Adediran, E., Olanrewaju, H. O., ... & Odumosu, J. (2022). The interrelationship between LST, NDVI, NDBI, and land cover change in a section of Lagos metropolis, Nigeria. Applied Geomatics, 14(2), 299-314.

Argüeso, D., Evans, J. P., Fita, L., & Bormann, K. J. (2014). Temperature response to future urbanization and climate change. Climate dynamics, 42, 2183-2199.

Athul, P. P., Sreelatha, A. K., Beena, V. I., Thomas, D., Lekshmi, V. A., Vijayan, V. D., Krishnan, A., & Parapurath, F. (2025). Spatial distribution of soil chemical properties and nutrient availability across different land use systems in Pokkali wetlands. Plant Archives, 26(1), 2476–2483.

Bai, X., McPhearson, T., Cleugh, H., Nagendra, H., Tong, X., Zhu, T., & Zhu, Y. G. (2017). Linking urbanization and the environment: Conceptual and empirical advances. Annual review of environment and resources, 42, 215-240.

Bonafoni, S., & Keeratikasikorn, C. (2018). Land surface temperature and urban density: Multiyear modeling and relationship analysis using MODIS and Landsat data. Remote Sensing, 10(9), 1471.

Bouhennache, R., Bouden, T., Taleb-Ahmed, A., & Cheddad, A. (2019). A new spectral index for the extraction of built-up land features from Landsat 8 satellite imagery. Geocarto International, 34(14), 1531-1551.

Boyaj, A., Nadimpalli, R., Reddy, D., Sinha, P., Karrevula, N. R., Osuri, K. K., ... & Kaginalkar, A. (2023). Role of radiation and canopy model in predicting heat waves using WRF over the city of Bhubaneswar, Odisha. Meteorology and Atmospheric Physics, 135(6), 60.

Chakrabarti, P. D. (2001). Urban crisis in India: new initiatives for sustainable cities. Development in practice, 11(2-3), 260-272.

Chapman, S., Watson, J. E., Salazar, A., Thatcher, M., & McAlpine, C. A. (2017). The impact of urbanization and climate change on urban temperatures: a systematic review. Landscape Ecology, 32, 1921-1935.

Chen, F., Yang, X., & Zhu, W. (2014). WRF simulations of urban heat island under hot-weather synoptic conditions: The case study of Hangzhou City, China. Atmospheric research, 138, 364-377.

Chen, Y. C., Chiu, H. W., Su, Y. F., Wu, Y. C., & Cheng, K. S. (2017). Does urbanization increase diurnal land surface temperature variation? Evidence and implications. Landscape and Urban Planning, 157, 247-258.

Das, N., Mondal, P., Sutradhar, S., & Ghosh, R. (2021). Assessment of variation of land use/land cover and its impact on land surface temperature of Asansol subdivision. The Egyptian Journal of Remote Sensing and Space Science, 24(1), 131-149.

Dou, P., & Chen, Y. (2017). Dynamic monitoring of land-use/land-cover change and urban expansion in Shenzhen using Landsat imagery from 1988 to 2015. International Journal of Remote Sensing, 38(19), 5388-5407.

Du, J., Fu, Q., Fang, S., Wu, J., He, P., & Quan, Z. (2019). Effects of rapid urbanization on vegetation cover in the metropolises of China over the last four decades. Ecological Indicators, 107, 105458.

Dutrieux, L. P., Jakovac, C. C., Latifah, S. H., & Kooistra, L. (2016). Reconstructing land use history from Landsat time-series: Case study of a swidden agriculture system in Brazil. International Journal of Applied Earth Observation and Geoinformation, 47, 112-124.

Falkner, R. (2016). The Paris Agreement and the new logic of international climate politics. International Affairs, 92(5), 1107-1125.

Fatemi, M., & Narangifard, M. (2019). Monitoring LULC changes and its impact on the LST and NDVI in District 1 of Shiraz City. Arabian Journal of Geosciences, 12(4), 127.

Fischer, E. M., Oleson, K. W., & Lawrence, D. M. (2012). Contrasting urban and rural heat stress responses to climate change. Geophysical research letters, 39(3).

Gogoi, P. P., Vinoj, V., Swain, D., Roberts, G., Dash, J., & Tripathy, S. (2019). Land use and land cover change effect on surface temperature over Eastern India. Scientific reports, 9(1), 8859.

Gouda, K. C., Sahoo, S. K., Samantray, P., & Himesh, S. (2017). Simulation of extreme temperature over Odisha during May 2015. Weather and Climate Extremes, 17, 17-28.

Gowtham, S., & Dhanapriya, M. (2025). Land use land cover (LULC) change detection of east & south eastern coastal plain of Odisha from 1990–2018 and its impact over land surface temperature (LST). J. Exp. Agric. Int., 47(6), 1-26.

Guleria, S., & Gupta, A. K. (2018). Heat wave in India documentation of state of Telangana and Odisha (2016). National Institute of Disaster Management, New Delhi, 124.

Güneralp, B., Reba, M., Hales, B. U., Wentz, E. A., & Seto, K. C. (2020). Trends in urban land expansion, density, and land transitions from 1970 to 2010: A global synthesis. Environmental Research Letters, 15(4), 044015.

Halder, B., & Bandyopadhyay, J. (2021). Evaluating the impact of climate change on urban environment using geospatial technologies in the planning area of Bilaspur, India. Environmental Challenges, 5, 100286.

Irfeey, A. M. M., Chau, H. W., Sumaiya, M. M. F., Wai, C. Y., Muttil, N., & Jamei, E. (2023). Sustainable mitigation strategies for urban heat island effects in urban areas. Sustainability, 15(14), 10767.

Karanam, H. K., & Neela, V. B. (2017). Study of normalized difference built-up (NDBI) index in automatically mapping urban areas from Landsat TN imagery. Int J Eng Sci Math, 8, 239-48.

Kumari, B., Tayyab, M., Shahfahad, Salman, Mallick, J., Khan, M. F., & Rahman, A. (2018). Satellite-driven land surface temperature (LST) using Landsat 5, 7 (TM/ETM+ SLC) and Landsat 8 (OLI/TIRS) data and its association with built-up and green cover over urban Delhi, India. Remote Sensing in Earth Systems Sciences, 1, 63-78.

Liaqut, A., Younes, I., Sadaf, R., & Zafar, H. (2019). Impact of urbanization growth on land surface temperature using remote sensing and GIS: a case study of Gujranwala City, Punjab, Pakistan. International Journal of Economic and Environmental Geology, 44-49.

Lohani, T. K. (2022). Causes, Effects, and Remedial Measures of Climate Change in the East Coast of India with Special Reference to the State of Odisha. In India II: Climate Change Impacts, Mitigation and Adaptation in Developing Countries (pp. 383-406). Cham: Springer International Publishing.

Macintyre, H. L., Heaviside, C., Taylor, J., Picetti, R., Symonds, P., Cai, X. M., & Vardoulakis, S. (2018). Assessing urban population vulnerability and environmental risks across an urban area during heatwaves–Implications for health protection. Science of the total environment, 610, 678-690.

Malik, M. S., Shukla, J. P., & Mishra, S. (2019). Relationship of LST, NDBI and NDVI using landsat-8 data in Kandaihimmat watershed, Hoshangabad, India. Indian Journal of Geo Marine Sciences, 48(1): 25- 31

Marzban, F., Sodoudi, S., & Preusker, R. (2018). The influence of land-cover type on the relationship between NDVI–LST and LST-T air. International Journal of Remote Sensing, 39(5), 1377-1398.

Meena, A. K., Meena, H., Parapurath, F., & Shanmugam, A. (2026). Rainfall distribution patterns during the Southwest Monsoon in Kota District, Rajasthan (2018–2023) and their impact on crop production. EQA-International Journal of Environmental Quality, 72, 65-73.

Meer, M. S., & Mishra, A. K. (2020). Remote sensing application for exploring changes in land-use and land-cover over a district in Northern India. Journal of the Indian Society of Remote Sensing, 48, 525-534.

Mishra, M., Santos, C. A. G., do Nascimento, T. V. M., Dash, M. K., da Silva, R. M., Kar, D., & Acharyya, T. (2022). Mining impacts on forest cover change in a tropical forest using remote sensing and spatial information from 2001–2019: A case study of Odisha (India). Journal of Environmental Management, 302, 114067.

Mishra, S. P., Sethi, K. C., Swain, N. M.., Behera, S. S., Siddique, M. (2020). Anthropocene Water-Body Changes in Coastal Towns, Puri, Odisha: Micro-Scale Geospatial Analysis. Indian Journal of Natural Sciences, 10 (59): 18190- 18200

Mohajerani, A., Bakaric, J., & Jeffrey-Bailey, T. (2017). The urban heat island effect, its causes, and mitigation, with reference to the thermal properties of asphalt concrete. Journal of environmental management, 197, 522-538.

Molnár, G. (2016). Analysis of land surface temperature and NDVI distribution for Budapest using Landsat 7 ETM+ data. Acta climatologica et chorologica, 49, 49-61.

Motesharrei, S., Rivas, J., Kalnay, E., Asrar, G. R., Busalacchi, A. J., Cahalan, R. F., ... & Zeng, N. (2016). Modeling sustainability: population, inequality, consumption, and bidirectional coupling of the Earth and Human Systems. National Science Review, 3(4), 470-494.

Nagendra, H., Bai, X., Brondizio, E. S., & Lwasa, S. (2018). The urban south and the predicament of global sustainability. Nature sustainability, 1(7), 341-349.

Nageswararao, M. M., Sinha, P., Mohanty, U. C., & Mishra, S. (2020). Occurrence of more heat waves over the central east coast of India in the recent warming era. Pure and Applied Geophysics, 177, 1143-1155.

Nayak, S. K., & Nandimandalam, J. R. (2023). Impacts of climate change and coastal salinization on the environmental risk of heavy metal contamination along the odisha coast, India. Environmental Research, 238, 117175.

Neethu, C., Ramesh, K. V., & Shafeer, K. B. (2020). Understanding the spatio-temporal structure of recent heat waves over India. Natural Hazards, 102(2), 673-688.

Parapurath, F., Rath, B. S., & Manikandan, N. (2020). Forest cover classification and change detection using NDVI over 1990 to 2020 in the Western Central Table Land Zone of Odisha. Forest, 745(2592.5), 3970-91.

Parapurath, F., Senapati, R., Ghosh, A., Rath, B. S., & Verma, D. (2025b). Spatio-temporal changes of the forest cover in North Eastern Ghat Zone of Odisha, India, using multi-year Landsat data. Nature Environment and Pollution Technology, 24(3), B4291.

Parapurath, F., Veluswamy, K., Rajan, R., & Prakash, A. (2025c). MONITORING AND PREDICTING DROUGHT DYNAMICS IN COIMBATORE USING SATELLITE DERIVED PDSI AND SPEI INDICES: A MACHINE LEARNING APPROACH. In 4th World Irrigation Forum (pp. 212). International Commission on Irrigation and Drainage (ICID).

Parapurath, F., Waquar, T., Lalmuanzuala, B., & Prakash, A. (2025a). GLOBAL WARMING SOLUTIONS: HARNESSING AI, ML & ADVANCED TECHNOLOGIES FOR A SUSTAINABLE FUTURE. AgriGate-An International Multidisciplinary e-Magazine, 5(9), 826-840.

Peng, J., Ma, J., Liu, Q., Liu, Y., Li, Y., & Yue, Y. (2018). Spatial-temporal change of land surface temperature across 285 cities in China: An urban-rural contrast perspective. Science of the Total Environment, 635, 487-497.

Prakash, A., Parapurath, F., Nazar, S., Meena, A. K., Panchulakshmi, M., & Waquar, T. (2025). APPLICATION OF GIS TECHNOLOGY FOR RESTORATION OF GROUNDWATER RESOURCES: A COMPREHENSIVE REVIEW. Plant Archives, 25(2), 2127-2136.

Roy, P. S., Ramachandran, R. M., Paul, O., Thakur, P. K., Ravan, S., Behera, M. D., ... & Kanawade, V. P. (2022). Anthropogenic land use and land cover changes—A review on its environmental consequences and climate change. Journal of the Indian Society of Remote Sensing, 50(8), 1615-1640.

Senapati, R., Rath, B. S., Parapurath, F., Mahanty, M., Ghosh, A., Meena, A. K., & Pandit, R. (2024). Forest Cover Change Detection Over North Eastern Ghat Zone of Odisha, India Using Multi-Year Landsat Data. International Journal of Environment and Climate Change, 14(9), 787-795.

Sidiqui, P., Huete, A., & Devadas, R. (2016). Spatio-temporal mapping and monitoring of Urban Heat Island patterns over Sydney, Australia using MODIS and Landsat-8. In 2016 4th International Workshop on Earth observation and Remote Sensing applications (EORSA) (pp. 217-221). IEEE.

Souza Jr, C. M., Z. Shimbo, J., Rosa, M. R., Parente, L. L., A. Alencar, A., Rudorff, B. F., ... & Azevedo, T. (2020). Reconstructing three decades of land use and land cover changes in brazilian biomes with landsat archive and earth engine. Remote Sensing, 12(17), 2735.

Subba Rao, A. V. M., Parapurath, F., Sarath Chandran, M. A., Bal, S. K., Manikandan, N., & Singh, V. K. (2025). Spatiotemporal variability and trends of hailstorms over India. Natural Hazards, 121(2), 1687-1710.

Tam, B. Y., Gough, W. A., & Mohsin, T. (2015). The impact of urbanization and the urban heat island effect on day to day temperature variation. Urban Climate, 12, 1-10.

Van Vuuren, D. P., Stehfest, E., Gernaat, D. E., Doelman, J. C., Van den Berg, M., Harmsen, M., ... & Tabeau, A. (2017). Energy, land-use and greenhouse gas emissions trajectories under a green growth paradigm. Global environmental change, 42, 237-250.

Virtudes, A., Abbara, A., & Sá, J. (2017). Dubai: A pioneer smart city in the Arabian territory. In IOP Conference Series: Materials Science and Engineering (Vol. 245, No. 5, p. 052071). IOP Publishing.

Wang, J., Feng, J., & Yan, Z. (2015). Potential sensitivity of warm season precipitation to urbanization extents: Modeling study in Beijing‐Tianjin‐Hebei urban agglomeration in China. Journal of Geophysical Research: Atmospheres, 120(18), 9408-9425.

Wei, Y., Chen, H., & Huang, J. J. (2024). Response of surface energy components to urban heatwaves and its impact on human comfort in coastal city. Urban Climate, 54, 101836.

Wu, C., Li, J., Wang, C., Song, C., Chen, Y., Finka, M., & La Rosa, D. (2019). Understanding the relationship between urban blue infrastructure and land surface temperature. Science of the Total Environment, 694, 133742.

Yan, Z. W., Wang, J., Xia, J. J., & Feng, J. M. (2016). Review of recent studies of the climatic effects of urbanization in China. Advances in Climate Change Research, 7(3), 154-168.

Zhao, N., Jiao, Y., Ma, T., Zhao, M., Fan, Z., Yin, X., ... & Yue, T. (2019). Estimating the effect of urbanization on extreme climate events in the Beijing-Tianjin-Hebei region, China. Science of the total environment, 688, 1005-1015.