Original Research Article

Impact of Grid Parity on the Integration of Renewable Energy in Cameroon’s Electricity Market

# ABSTRACT

The integration of renewable energies becoming crucial in the electricity market. Cameroon has set the target of achieving 25%  share of renewables in its energy mix by 2030. The aim  of this study is to analyze the impact of grid parity on Cameroon’s electricity market. The methodology involved conducting an assesment of Cameroon’s electricy system , then, using a questionnaire and open-ended interviews with electrical engineering experts, identifying the factors that hindering grid parity, and carrying out an analysis using quality tools such as the Ishikawa diagram, SWOT, PESTEL and Pareto diagram, to finally develop a strategic action plan using the 3P framework. The results show that from 2019 to 2024, the integration rate ranges from 0.04% to 0.84% according to the 2011 law governing Cameroon’s electricity sector on renewable energies, and under the same conditions, from 2024 to 2030, the rate would evolve according to three scenarios, the high scenario at 3%; the median scenario at 2% and the low scenario at 1%. Based on feedback from the experts, 59 causes have been identified, including 21 weaknesses and 12 threats. The predominant causes are technical or technological, social, economic and political factors, which account fort 80% of the factor influencing the system and, consequently,  20% of the barriers  to grid parity. We have therefore  proposed a strategic action plan comprising  four priority actions; (i) popularising techniques and technologies, (ii) strengthening the participation of social stakeholders  in the planning and management of renewable energies, (iii) promoting mechanisms for procurement of green installations, (iv) updating national policy.

Keywords: Grid parity, renewable energy, integration, Cameroon’s electricity market.

- INTRODUCTION

A balance between supply and demand is the best way to ensure that the electricity market competitive and attractive to investors. It lays the foundation for a strategic approach that  enables energy policy and climate policy to be aligned in the context of the energy transition. Indeed, the concept of green growth has become indispensable today for any development policy, and encompasses several concepts that bring new challenges, such as the circular economy, sustainable development, and energy efficiency (Guergaziz, 2021).

In 2011, the European Commission, together whit its partners launched the "PV Parity" project to propose to national policy-makers in 11 European Union countries (Austria, Belgium, the Czech Republic, France, Germany, Greece, Italy, the Netherlands, Portugal, Spain and the United Kingdom) with supporting measures to achieve grid parity and beyond . The project report concluded that photovoltaics sector is increasingly shifting from an investor-driven market towards a market of responsible consumers. This shift can be explained by the transition from a market based on feed-in tariffs (Feed In Tariff) to support mechanisms centred on self-consumption (Clastres & Rebenaque, 2020).

In 2015, the work of Hugo Lequertier, Ouassat Abderazak, Rogissart Lucile and Sandevoir Claire highlighted  the threats posed to the electricity market by renewable energies and suggested some possible solutions. In the case of the European Union, these involve: (i) adapting the energy system with greenhouse gas (GHG) emission quotas; (ii) setting meaningful prices for carbon dioxide (CO2) emissions; (iii) building a network that allows renewable energy to be fully integrated int the common market; (iv) reducing  the price of electricity in the event of overproduction due to renewable energy; (v) granting subsidies for renewable energy as necessary temporary support; (vi) achieving grid parity for photovoltaics. This would enable individuals and businesses who so wish to invest in renewable electricity generation without the need state aid (Lequertier et al., 2015). However, the massive injection of renewable electricity into the interconnected grid  has crowding out effects and requires storage systems to avoid jeopardizing the balance of the interconnected grid. This is why it is desirable for the share of renewable energy in the electricity mix does not exceed 30%, to allow solar electricity to approach grid parity (Percebois & Pommeret, 2018).

Electricity  generation from so-called new renewable sources is now a priority in international climate commitments. Cameroon has strong potential in renewable energy, especially in hydropower and solar, which could support universal access to energy while reducing emissions, it can undertake a credible sustainable energy transition, but only if public policies strongly support renewables, rural electrification and the resilience of the energy system (Manjong & al., 2021). In the case of Cameroon, this is underpinned by the National Development Strategy to 2030 (SND 30), particularly the energy and waste component, which aims to develop energy production from renewable sources and whose objective is to increase the share of renewable energy (excluding large-sacle hydro) in the electricity mix to 25% by 2035.

- MATERIALS AND METHOD

Our study is mixed-methods. On the one hand, it is qualitative, employing the chosen data analysis method-semantic and structural content analysis (N. LEFEVRE, 2013) which is suitable for research, the analysis of reports, and the identification of relationships between key concepts withing a dataset. of a dataset. and on the other hand, quantitative , using descriptive statistics to summarise and describe a dataset using tables, graphs and measures such as the mean, median or standard deviation. It also take a forward-looking view from 2025 to 2030.

Sampling was carried out using the judgemental or purposive sampling method,  as it only selects only those individuals who meet the research criteria and final objectives, whilst others are excluded, with as overall population comprising experts from the Order of Electrical Engineering Engineers. The formula for the sample size (N).

Sample size=Z² x p x  1-pc²

Sample size=Z² x p x  1-pc²

Where:

Z2 = confidence level (95% or 99%);

P = 0.5;

C2 = Margin of error (0.04 =   4%).

N greater than 91 ( N>91). Data entry, processing and analysis were carried out using Excel sofware , Power BI, and online tool https://www.questionpro.com/sample-size-calculator/. In order to identify the causes that affecting grid parity, the experts were interviewed using semi-structured and open-ended, after which a cause tree was developed, the impact of the causes was analysed using SWOT, they were grouped by factor using PESTEL, prioritise them using Pareto (80/20) method, and finally structure the strategic action plan around the 3Ps (implementation, monitoring of effectiveness and tracking of efficiency).

- RESULTS AND DISCUSSION

- RESULTS 1: CURRENT SITUATION

  - ELECTRIC MIX OF ACTIVITIES

The electricity mix of activities takes into account all power stations operated by electricity sector operators that are subject to regulatory principles, as well as those built by third parties. The figures below indicate the rates of primary sources that make up the electricity mix.

Figure 1: Electrical mix of regulated activities (Source : ARSEL,2025)

For activities of the electricity sector including those of third parties, in 2025 hydro and thermal sources will account for 97% of the total installed capacity, with solar photovoltaic accounting for 3%.

  - CHANGES IN THE AVERAGE TARIFF WHITHIN THE ELECTRICITY SYSTEM

The analysis of data collected regarding energy sold from 2019 to 2023 allowed us to highlight the evolution of energy and average price. The figure below illustrates the evolution of the tariff based on the energies sold from 2019 to 2023.

Figure 2: Evolution of the average tariff and energy sold. (Source : ARSEL,2025)

The average tariff is generally the ratio of the amount of sales in FCFA to the quantity of energy sold in kWh. The evolution of the average tariff is not linear, so other specific parameters are involved in the intermediate calculations. We can mention technical and non-technical or commercial losses, frauds, non-payment of electricity bills, estimates, etc. Overall, there is a clear increase in the quantity of energy sold from 2019 to 2023.

  - CHANGES IN THE THE AVERAGE TARIFF BY SOURCE OF GENERATION

From the analysis of the data collected from 2019 to 2023, we have categorized by type of production source and highlighted the evolution of the average tariff. Figure 3 summarises the results obtained.

Figure 3: Evolution of the average tariff by production source. (Source : ARSEL,2025)

The evolution of the average electricity system tariff in Cameroon from 2019 to 2023 is identical for all production sources. the annual average rates are in slope, the highest average rate was observed in 2019 and the lowest average rate in 2021. The causes can be related to investments, to the evolution of demand, to population growth. With the usual pricing model in Cameroon, 'the service cost' and a billing structure designed to have a single kWh rate throughout the national territory for each type of consumer, to be competitive in this case the average renewable energy tariff must be less than 74 FCFA.

  - CHANGES IN THE SHARE OF RENEWABLE ENERGIES IN THE ELECTRICITY MIX

The energy transition is at the heart of the electricity sector and the integration of renewable energies is taking place at a very slow pace. It is enough to observe the curve of the evolution of the integration of renewable energies into the energy mix on the basis of sold energies, which corresponds to an affine function written in the form:

f (x ) = ax+b. with two conditions, , a et b  R et, and et b ≠ 0, where: a is the director coefficient, and b the intercept;

Figure 4: ENR weights and projection to 2030.

An analysis of data on energy sale volumes from the distribution operator ENEO shows that, from 2020 onwards, the generation of electricy from so-called renewable energy sources will becomes a reality to helping to reduce fuel costs which are projectd to reach 42 billion in 2023 and 65 billion, to be accounted for to the Regulator for the 2024  financial year  and gradually phase out thermal energy production using fuel oil, whic is harmful to our environment. To date, in the present case, the share  of renewable energy does not even reach 1%. This figure does not take into account hydroelectric production of less than 5MW, in accordance with the 2011 Law governing the electricity sector, nor does it account for all error in data colection and processing. If the pace of implementing renewable energy integration is not accelerated, three final scenarios are projected for the period 2025 to 2030. In the high end scenario, the figure is expected to  reach 3%; the medium end scenario, 2% ; and in the low end scenario, it will remain around 1%.

- RESULT 2: ANALYSIS OF FACTORS HINDERING THE ACHIEVEMENT OF GRID PARITY IN CAMEROON

The following table 1 seta out the main factors identified based on the questionnaire and interview protocol. The majority of the experts consulted for our study are menbers of the National Order of Electrical Engineers.

Table 1: Results of the identified causes.

| N° | Identified causes |
| --- | --- |
| 1 | Fraud |
| 2 | Public subsidies |
| 3 | Tariff compensations |
| 4 | Recovery problem |
| 5 | Structural reforms |
| 6 | Aging infrastructure |
| 7 | Poor quality of service |
| 8 | Liberalization of the sector |
| 9 | Consumer mentality |
| 10 | Electricity price |
| 11 | Insufficient ENR-specific standards |
| 12 | Legal provisions relating to renewable energies |
| 13 | Regulatory framework specific to ENR |
| 14 | Regulation not adapted to the promotion of ENR |
| 15 | Unsuitable ENR energy policy |
| 16 | International agreements |
| 17 | Low purchasing power |
| 18 | Reduced tax policy for renewable energies |
| 19 | vulnerability to climate change |
| 20 | Dependence on hydroelectric dams |
| 21 | Dependence on thermal energies based on fuel |
| 22 | no access to electricity in rural areas |
| 23 | Ambitious Governmental Strategic Goal |
| 24 | Bad hydrology |
| 25 | Untapped ENR potential |
| 26 | Increase in energy demand |
| 27 | Favourable political framework |
| 28 | Support for international funding |
| 29 | Diversification of the energy mix |
| 30 | Energy security |
| 31 | Dependency on hydraulics |
| 32 | Population growth |
| 33 | Large hydroelectric projects costly and long to implement |
| 34 | Import of petroleum products for thermal production |
| 35 | Low electrification rate |
| 36 | Electricity Act 2011 |
| 37 | Program and initials of the funders |
| 38 | Public Private Partnerships |
| 39 | Local industry |
| 40 | Green jobs |
| 41 | Global decline in solar costs |
| 42 | Fossil fuel subsidies |
| 43 | Complex tariff structure |
| 44 | Use of fossil thermal power plants |
| 45 | Price differentiation |
| 46 | Limit of use of energy-consuming devices |
| 47 | Limited knowledge of technology |
| 48 | Extension of the limited network |
| 49 | Distance from consumers to the network |
| 50 | Untimely power outages |
| 51 | Possibility of installation on the roofs of buildings to avoid land or land use conflicts |
| 52 | Corruption |
| 53 | Energy poverty |
| 54 | Local opposition due to concerns about the landscape |
| 55 | Helping the most vulnerable households |
| 56 | Training and requalification needs for workers in the fossil fuel sectors |
| 57 | Insufficient skilled labor |
| 58 | Pollution from heating and cooking |
| 59 | Lack of solidarity and collective actions. |

It appears that 59 causes are  responsible for the low  integration of renewable energy into the energy mix. Based on the results of the cause tree, the following table presents the results of the analysis carried out that out to identify the external and internal influences of each cause.

Table 2: SWOT analysis of the causes.

| strengths | Weaknesses |
| --- | --- |
| - Public subsidies - Structural reforms - Electricity price - Legal provisions relating to renewable energies - Regulatory framework specific to ENR - Reduced tax policy for renewable energies - Diversification of the energy mix - Energy security - Electricity Act 2011 - Local industry - Favourable political framework | - Fraud - Tariff compensations - Aging infrastructure - Poor quality of service - Insufficient ENR-specific standards - Dependence on hydroelectric dams - Dependence on thermal energies based on fuel - No access to electricity in rural areas 11. Low electrification rate 12. Fossil fuel subsidies 13. Complex tariff structure 14. Use of fossil thermal power plants 21.Local opposition due to concerns about the landscape |
| Opportunities | Threats |
| - LiIberization of the sector - Energy policy - International agreements - Ambitious Governmental Strategic Goal - Support for international funding - Donor program and initiatives - Public Private Partnerships - Green jobs - Global decline in solar costs - Possibility of installation on the roofs of buildings to avoid land or land use conflicts | - Large hydroelectric projects that are costly and take a long time to implement - Import of petroleum products for thermal production - Limited knowledge of technology - Corruptions - Lack of solidarity and collective action. - Vulnerability to climate change - Low purchasing power |

This SWOT analysis highlights 21 weaknesses and 12 threats. The following table present table presents the various causes by PESTEL factors, along with their respective weights :

Table 3: Causes identified by PESTEL factor.

|  | Political |  |
| --- | --- | --- |
|  | Cause | Weight |
|  | 7 | 12% |
|  | Economic |  |
|  | Cause | Weight |
|  | 9 | 15% |
|  | Social |  |
|  | Cause | Weight |
|  | 12 | 20% |
|  | Technical ou Technological |  |
|  | Cause | Weight |
|  | 19 | 32% |
|  | Environment |  |
|  | Cause | Weight |
|  | 6 | 10% |
|  | Legal |  |
|  | Cause | Weight |
|  | 6 | 10% |
| Total | 59 | 100% |

Thus,according to this table, the factor with the highest number of causes is the Technical or Technological factor, with 19 occurrences representing 32% of the factors influencing the national energy mix, followed by the Social factor with 12 occurrences, representing 20%, followed by the economic factor with 9 occurrences,or 15%,followed by the political factor with 7 occurrences or 12%, and finally the last two factors,Environmental and Legal, with 6 occurrences,or 10%. The resulting PARETO curve identifies the factors with the greatest impact.

Figure 6: Hiérarchisation des facteurs d'impacts de l'intégration des EnR dans le Mix énergétique

Thus, according to this figure, the Technical or Technological, Social, Economic and finally Political factirs represent the factors with the greatest impact on the system, as they account for 80% of the factors influencing the system and, to this end, represent 20% of the factors according to the Pareto principle. This means that if one we wish to improve the integration of renewable energy into the energy mix, we must address the causes associated with these factors. It therefore follows that the central problem of integrating renewable energy into the energy mix stems from technical or technological, social, economic and political factors, which manifest as  : (1) technology that is very stagnant and techniques that are still rudimentary; (2) a glaring lack of involvement from civil society; (3) low purchasing power and inadequate subsidies; (4) an insuitable national policy on renawable energy. The problems linked to these factors form the basis for strategic planning.

RÉSULT 3 : STRATEGIC ACTION PLAN FOR THE INTEGRATION OF RENAWABLE ENERGY INTO COMEROON’S ELECTRICITY MIX

The strategic action plan is structured around Priority Action Areas (PAAs). The table below highlights four colour coded zone for each priority.

Table 4: Priority rating of actions

|  | Priority Action Area 1 Requiring a short-term response. Disseminate techniques and technologies related to the integration of renewable energy into the energy mix (PAA 1) |
| --- | --- |
|  | Priority Action Area 2 Requiring a short- or medium-term response. Strengthen the involvement of social stakeholders in the planning and management of renewable energy at national level (PAA 2) |
|  | Priority Action Area 3 Which requires a medium-and long-term response. Promoting mechanism for the procurement of renewable energy facilities (PAA 3) |
|  | Priority Action Area 4 Which requires a response in the medium to long term. Update the national policy framework for renewable energy management (PAA 4). |

The following tables present the results obtained.

the estimated total cost of the strategic action plan is summarised in table 4 below. The details of the strategic plan are presented in the tables 6 at 8 in the appendix.

Table 5: Total cost of the Priority Action Areas of the Strategic Plan

| Area of Action |  | Estimated Cost |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Implement | Effectiveness | Efficiency | Total |
|  | Priority 1 | 121 000 000 | 35 000 | 10 000 | 121 045 000 |
|  | Priority 2 | 1 225 500 | 30 000 | 10 000 | 1 265 000 |
|  | Priority 3 | 18 620 000 | 35 000 | 15 000 | 18 670 000 |
|  | Priority 4 | 10 213 000 | 100 000 | 15 000 | 10 328 000 |
| Total Cost (KFCFA) |  | 151 058 500 | 200 000 | 50 000 | 151 308 500 |

The cost of implementing priority actions to achieve grid parity is estimated at  151,308 K FCFA for the period from 2025 to 2030.

The analysis of the impact of grid parity on the integration of renewable energy into the electricity market involved assessing and understanding the strategiy implemented by the Government of Cameroon to promote renewable energy.

The share of renewable energy will be less than 1% in 2024.  Analysis of the data indicates thress possible scenarios for 2025 to 2030, namely : lthe high scenario,where 3% could be achieved; the medium scenario,where 2% could be achieved ; andthe low scenario,where the figure could remain around 1%. This reflects the fact that investment in electricity generation from renewable energy sources is low.In the work by (EL Moummy et al., 2021) on the impact of renewable energy on economic growth in the case of Morocco, the key recommendation in terms of energy policy concern fiscal measures for renewable energy. Indeed, just as in Morocco in  2021, the situation in Cameroon in 2025 appears to us to be almost identical. The Cameroonian government, as the Moraccan government has done since 2021, must encourage the use of renewable energy sources over conventional ones by introducing tax incentives to stimulate investment and consumption, and by facilitatting the transition from thermal energy to green technologies. On the other hand, it is necessary to create a climate conducive to investment, enabling the country to attract potential investors. The training of a skilled workforce and the development of human capital are also necessary condition for the deployment of the renewable energy sector,notably the renewable energy sector. Which has been included in the finance act since 2012, with the list deemed approved upon publication by the Minister of Finance. In practice, however, it must be noted that the customs administration require, in order to benefit from this exemption, that a request be submitted to the Director General of Customs. The administration is therefore creating a condition not provided for by law, which suggest that the General Directorate of Custom is acting contrary to the mechanisms established by the State to ensure the exemption (Zang & Ozoto, 2024).We understand that, in view of the incentives adopted by the Government of Cameroon to promote renewable energy, this is a quantity-based innovation (Bourgeois et al., 2017), as the low-voltage electricity tariff in cameroon ha not changed since 2012, thereby confirming the 1% intégration rate for renewable energy and justifying the view that grid parity cannot be achieved over the next five years. des énergies renouvelables, et justifie que la parité.

The identification, analysis, classification and prioritisation of cause using quality tools has enabled the identification of the 20% of factors responsible for 80% of the problems related to the failure to achieve grid parity in cameroon’s energy mix. These include technical or technological,social, economic and political factors. By addressing these factors,Cameroon’s electricity system can achieve grid parity. Renewable energy can become a threat to the electricity market once they account 30% of the electricity mix ; solutions include : (i) aligning the energy system with the European Union’s greenhouse gas (GHG) emission quotas; (ii) setting meaningful prices for carbon dioxide (CO2); (iii) building a grid that allows renewable energy to be fully integrated into the common energy market; (iv) reducing the price of electricity in the event of overproduction due to renewable energy; (v) granting subsidies to renewable energy as necessary temporary support to allow thes technologies to mature (Lequertier et al., 2015), are among the factors responsible for grid parity not being achieved.

In the scenario, the strategic plan sets out the action to be taken to achieve grid parity, based on the key factors identified in the PESTEL analysis. For the period from 2025 to 2030, four (04) priority actions have been proposed to achieve grid parity. The case of Ageria’s renewable energy strategy, which has established a two-satge renewable energy development programme. From 2015 to 2020 : a capacity of 4,010 MW will be achieve through photovoltaic and wind powerser, as well as 515 MW through biomass, combined heat and power, and geothermal energy ; and from 2021 to 2030 : the installation of large-scale renewable energy power stations in the regions of in salah, Adrar, Timimoune et Bechar ans their integration into the national energy system in oder to develop the electricity interconnection between the North and the Sahara. These two phases were preceded by a pilot phase from 2011 to 2014, during which several projects were carried out, and in 2016, a new strategy was  introduced by the Ministry of Energy for the development of renewable energy. Implementation is governed by Executive Decree N°17-98 du 26 February 2017 .  thus, the five (05) years implementation period set out in our strtagic plan linked to this study correponds to that of Algeria’s pilot phase, during which the country invested in the devlopment of approximately 162 MW (Guergaziz, 2021). Consequently, within the same timeframe, it will not be possible to achieve the results envisaged by the strategic priorities set out in the plan.

Overall, it is clear that, to provide an effective response to the challenges of integrating renewable energy in Cameroon, solutions must be combined in propotions that respect the context and the enviornment. For example, the addendum to annex 3 of ENEO’s framework concession contract defines the following zones : metropolitan areas, defines the following zonesdu contrat cadre de concession de ENEO définit les zones : métropolitaines areas, notably  Douala and Yaoundé, urban areas; all towns with more than 50,000 inhabitants, and rural areas,comprising all parts of the territory not included in metropolitan or urban areas. Taking this classification into account,it is possible to combine solution in ways that will have a significant impact on achieving grid parity. It is understood that grid parity. It is understood that grid parity can be broken down by the production cost of each electricity grid. In any case,based on the findings of this study ,we emphasise that grid parity in Cameroon’s electricity market will not be achieved between 2025 and 2030 ; however,there will be a slight increase in the share of renewable energy in the electricity mix due to third-party installation.

- CONCLUSION

Electricity generation in Cameroon from renewable energy sources is not optimised. This means they account for a negligible share of the electricity market. Achieving grid parity remains one of the major challenges for the integration of renewable energy in Cameroon, given its impacts. The overall objective of the article was to conduct a study of the impact of grid parity on the integration of renewable energy into Cameroon’s electricity market. It is essential to note that gris parity is the situation in which the cost of renewable energy sources becomes as competitive as that of other energy sources within a given electricity system is the primary target to be achieved for the promotion of renewable energy. This study was conducted over a period of six  (06) months, from February to august 2025. Interviews conducted with members of the National Order of Electrical Engineers reveal a multitude of factors hindering the achievement of gris parity in Cameroon.

The result show that, to date, renewable energy is developing according to three possible scenarios for 2025 to 2030: the hugh-end scenario 3%; the medium-end scenario at  2% ; and the low-end scenario at around 1%. Based on the feedback, 59 causes have been identified, 21 of wich are weaknesses and 12 threats to achieving grid parity. The analysis highlights that the major causes stem from technical or technological,social,economic and political factors which account 80 % of the factors influencing the system and ,consequently, 20 % of the barriers to be overcome . based on these results, a strategic action plan has been proposed, implementing four priority actions: (i) popularising technique and technologies related to the integration of renewable energy, (ii) strengthening the involvement of social stakeholders in the planning and management of renewable energy, (iii) promoting mechanisms for the procurement of renewable energy installation, (iv) updating the national policy for renewable energy management with estimated cost of 151,308,000,000 CFA franc.

This study confirms the importance of grid parity as a key factor for the successful promotion of renewable energy. By identifying challenges, capitalising on opportunities and mobilising key stakeholders in the electricity sector to take sustainable measures within the competitive environment of the electricity market

- LIMITATION

Limited access to data, the emerging development of the renewable energy market outside hydropower, the constraints imposed by the national power grid and the stagnation of legislation and regulation in this sector constitute obstacles to our research.

Funding and AUTHORSHIP CONTRIBUTION STATEMENT

Georges Franklin ONIASSEN MOUTOK: Conceptualization, Investigation, Methodology, Formal analysis, Data curation, Validation, Funding acquisition, Project administration, writing original draft.

Armel Albert EBANGO EWOUNDJO et Severin MBOG MBOG: Methodology, Data curation, Validation.

Dieudonné BITONDO and Ruben Marcel MOUANGUE  :

Supervision, Methodology, Data curation. Validation.

Competing Interest

The authors declare that they have no known competing financial interests or personal relationships that could have appeared to influence the work reported in this paper.

Acknowledgments

The authors sincerely thank the Laboratory of Energy, Materials, Modelling and Methods, of Higher National Polytechnic School of Douala, University of Douala, P.O. BOX 2701, Douala, Cameroon for the facilities granted in the accomplishment of this work.

# REFERENCES

1.	AIE, & OCDE. (2020). Projected costs of electricity generation. OECD Publishing. https://doi.org/10.1787/a6002f3b-en.

2.	ARETA Academy. (2020a, juin 25). ARETA (AFRICA RENERGY E-TRAINING ACADEMY) [GreenFiance  SMART GRID]. What is grid parity?.

3.	ARETA Academy. (2020b, june 25).Green Finance [Blog]. What is grid parity?

4.	ARSEL. (2022). History of the electricity sector in cameroon. Newsletter.

5.	Bardin, L. (2001). Content analysis. PUF.

6.	Biboum, A., & Yilanci, A. (2021, Aout). Thermodynamic and economic assessment of solar thermal power plant for Cameroun. Journal of solar energy engineering.

7.	Bourgeois, G., Mathy, S., & Menanteau, P. (2017). The effect of climate policies on renewable energy : A review of econometric studies. HAL Id:hal-01585906, 15‑39.

8.	CFI Éducation inc. (s. d.). Levelised Cost of Energy (LCOE) [Blog]. Levelised Cost of Energy (LCOE) . https://corporatefinanceinstitute.com/

9.	Clastres, C., & Rebenaque, O. (2020). The impact of grid pricing and support policies on the development of photoviltaic self- consumption [UNIVERSITy OF GRENOBLE ALPES]. https://theses.hal.science/tel-02975581v1

10.	Commune verte. (2025, janvuary 1). Grid arity [Https://communes-vertes.org/reseaux-ee/knowledge-hub/solaire-thermique/parite-reseau/]. Grid parity.

11.	Tariff Decision N°0000096/ARSEL/DG/DCEC/SDCT, Pub. L. No. Décision N°0000096/ARSEL/DG/DCEC/SDCT, 3 (2012).

12.	decree n°2015/454 du 08 october 2015 establishing the National Electricity Transmission Compagny, Pub. L. No. 454, n°2015 Decree (2015).

13.	EL Moummy, C., Salmi, Y., & Baddih, H. (2021, 08). The impact of renewable energy on economic growth:An emprirical analysis of the Morocan case.

14.	ESSAME, P. (2024). The electricity sector in Camerons.

15.	Essame, P. (2024). Strategy for succeful transition from post-paid to pre-paid electricity billing: the case of Cameroon's low-voltage customers  [Specialised Master's Thesis in Regulation]. Paris school of Mines.

16.	Guergaziz, W. (2021). Renewable energy : A cornestone of sustainable development in Algeria. international Journal of Advanced reseach on Planning and Sustainable development, 4(1), 46‑68.

17.	Guillard, Q. (2024). regulated tariff in the electricity market [PhD Thèsis]. Doctoral School of Law.

18.	Hendje, M., & Zang, A. D. (2024). The tax advantage of public-private partnership contract in Cameroon. PHONIX ADVISORY. https://phoenixadvisory-cm.com/2024/11/the tax advantage of public-private partnership contract in Cameroon./

19.	Jamiletou, M. (2024). Analysis of quality performance criteria for small and medium-sized entreprises in the  Douala iv. [Master's Resaech Thesis]. Doctoral School of Fundamental and Applied Sciences, University of Douala.

20.	LEFEVRE, N. (2013). Survey methods and techniques.

21.	Lequertier, H., Ouassat, A., Rogissart, L., & Sandevoir, C. (2015, may 18).Is the European electricity market threatened by renewable energy? 18.05.2015, 3.

22.	Mbodiam, B. (2024, may 3). Investing in Cameroon. Renewable energy : Cameroon unveils its strategy to install 1,500 MW by 2035.

23.	Menga, F. D. (2025a, march 12). Sizing of off-grid  PV systems.

24.	Menga, F. D. (2025b, march 12). Overview of solar energy-concersion technologies.

25.	Meunier, F. (d.).renewable Energy (Le cavalier Bleu). Le cavalier Bleu.

26.	MINEE (2011).

27.	MINEPAT. (2020). National Development Strategy.

28.	Monjoie, L. (2024). Securing Investments in Electricity Markets. Three Essays on Market Design [Economics and Finance, Université Paris sciences et lettres]. https://theses.hal.science/tel-04608818

29.	Odoroaga, M. (2016, march 10). Types of Renewable Energy. ALAMY.

30.	Percebois, J., & Pommeret, S. (2018a, january 26). Massive injection of intermittent and fatal renewable electricity into an interconnected grid: Crowding-out effects and storage requirement. Center for reseach in Energy Economics and law (CREDEN), 13.

31.        Manjong,A.S,Oyewo, C.Breyer (2021). Setting the pace for a sustainable energy transition in Central Africa: the case of Cameroon.

32.	Percebois, J., & Pommeret, S. (2018a, january 26). Massive injection of intermittent and fatal renewable electricity into an interconnected grid: Crowding-out effects and storage requirement. Center for reseach in Energy Economics and law (CREDEN), 14.

33.	Pilo, E. (2018). Zoning of the electricity distribution segment envisaged by 2031 in Cameroon : Conditions for sucess. [Master's thesis]. MINES ParisTech.

34.	Zang, A. D., & Ozoto, J. D. (2024). The customs exemption on solar energy equipment and materials granted by the  2024 Finance Act:A practical assessment  . PHONIX ADVISORY. https://phoenixadvisory-cm.com/2024/08/ customs-exemption-on-solar-energy-equipment-and-materials-granted-by-the-2024-finance-law-in-practice//

Appendix.

Table 6 : implementation plan for the Priority Action Areas

| PAA | Areas of intervention | Operational Objectives | Activities relating to the implementation of the area of intervention | Period | Actors of implementation | Cost (KF CFA) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PAA 1 | Establish moderm training centres specialising in renewable energy trades. | Construction of a trainin center per municipality,i.e 360 training centres across the country. | - Identification of academic institution capable of establishing moderm training center for renewable nergy professions. - Establishment of practical laboratories for renewable energy professions. | 2 years | MINEE, MINFOP, MINJEC, MINESUP, et their constituent bodies, Higher education, universities, reseachers, etc. | 43,000,000 |  |
|  | Construction of production and/or assembly plants for renewable energy equipment. | Estalishment of at least two factories per province, i,e. 20 factories | - Identification of renewable energy equipment for our industries. - Incorporate projets for the construction of production and/or assembly plants for renewable energy equipment into the Regional Council’s plans | 2 years | MINEE, MINMID, MINRESI, MINTP MINDUH,MINCAF,MINDDEVEL. Experts, etc. | 78,000,000 |  |
|  | Overall Cost |  |  |  |  | 121,000,000 |
| PAA 2 | Compile a list of civil society expert in renewable energy and promote their work . | Compilation of list of civil society representatives with a good understanding of issues relating to renewwable energy,covering the whole country. | - Identify civil society experts in renewable energy and allocate them to cover all regions, departements and municipalities. - Publish the list of these Experts. | 2 ans | MINDDEVEL,MINEE,MINEPDED, MINPMEESA | 1,225,500 |  |
|  | Overall Cost |  |  |  |  | 1,225,500 |
| PAA 3 | Introduce the green energy voucher and make tax and customs exemptions more acessible. | Identification of all authorised credit institution in Cameroon and lendersthat can issue green voucher. | - Definition of the sector of activity eligible for green voucher. - Formulation of energy packages adapted to consumer needs by sector of activity. | 3 ans | Commercial banks, Micro – finance, FEICOM, MAETUR, CFC,SNI | 6,1700,000 |  |
|  | Intégrating slora installation as a civic contribution. | Adopting a pilot programme to create energy « prosumers ». | - Incorporate solar installation as an option with the distribution concessionaire. - Incorporate « prosumer » into certain construction projects . | 5 ans | ENEO, AER, MINEE, MINEPAT, ARSEL. | 12,000,000 |  |
|  | Overall Cost |  |  |  |  | 18,620,000 |
| PAA 4 | Accelerating reform in the electricity sector. | Drafting of implementing regulations covering all technicla aspects of revewable energy. | Proposing draft implementing decrees for renewable energy | 5 ans | All stakeholders in the electricity sector | 10,213,000 |  |
|  | Overall Cost |  |  |  |  | 10,213,000 |

Table 7: Plan formonitoring the effectiveness of the implementation of the Priority Action Areas

| PAA | Areas of intervention | Activities relating to the implementation of the area of intervention | Effectiveness monitoring period | Effectiveness monitoring indicator | Means of verification | Costs (KF CFA) | Those responsible for or involved in monitoring effectiveness |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PAA 1 | Establish moderm training centres specialising in renewable energy trades. | Construction of training center per municipality, i .e 360 training centres across the conutry. | 01 year | Number of training center. | Reports | 20,000 | MINEE, MINFOP, MINJEC, MINESUP, MINCAF,MINTP,MINDDEVEL, MINPMEESA, FEICOM and partners |
|  | Construction of production and/or assembly plants for renewable energy equipment. | Estalishment of at least two factories per province, i,e. 20 factories . | 01 year | Number of factories opened | Reports | 15,000 | MINDDEVEL,MINTP, MINDUH,MINCAF,MINDDEVEL. |
|  | Overall Cost |  |  |  |  | 35,000 |  |
| PAA 2 | Compile a list of civil society expert in renewable energy and promote their work . | Compilation of list of civil society representatives with a good understanding of issues relating to renewwable energy,covering the whole country. | 06 months | Number of expert by region, department and municipality. | Publication of the list of experts | 30,000 | MINDDEVEL,MINEE,MINEPDED, MINPMEESA |
|  | Overall Cost |  |  |  |  | 30,000 |  |
| PAA 3 | Introduce the green energy voucher and make tax and customs exemptions more acessible. | Identification of all authorised credit institution in Cameroon and lenders that can issue green voucher. | 06 months | Number of credit institution identified | Reports | 15,000 | Commercial bank, Micro – finance, ,MINFI,MINEPAT,MINEE,MINNDEVEL,MINHDU. |
|  | Intégrating solar installation as a civic contribution. | Adopting a pilot programme to establish energy « prosumer ». | 01 year | Number of energy package | Guide | 20,000 | MINFI,MINEPAT,MINEE,MINNDEVEL,MINHDU. |
|  | Overall Cost |  |  |  |  | 35,000 |  |
| PAA 4 | Accelerating reform in the electricity sector. | Drafting of implementing regulations covering all technicla aspects of revewable energy. | 03 months | Number of implementing regulations available | Reports | 100,000 | ARSEL, MINEE, Electricity sector operators, Industrial clients, Consumer associations. |
|  | Overall Cost |  |  |  |  | 100,000 |  |

Table 8: Plan for monitoring the efficiency of priority Action Areas

| PAA | Areas of intervention | Expected Results | Baseline | Efficiency indicators | Method of verifying the indicator | Indicator verification period | Cost (KF CFA) |  | Stakeholders involved in monitoring the effectiveness of implementation |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PAA 1. | Establish moderm training centres specialising in renewable energy trades. | Establishment of modern training center specialising in renewable energy professions. | 01 center opened at the Faculty of Science in Yaoundé 1. | Number of training centres per year. | Report | 5 years | 5,000 |  | MINEE, MINFOP, MINJEC, MINESUP, and their dismemberments. |  |
|  | Construction of production and/or assembly plants for renewable energy equipment. | Construction of factories for the production and /or assembly of equipement for renewable energy.. | No factories. | Number of production and/or assembly plants opened. | Report | 5 years | 5,000 |  | MINSUP, MINESEC, MINRESI, Universities, higher education institutions, researchers, etc. |  |
|  | Overall Cost |  |  |  |  |  |  | 10,000 |  |  |
| PAA 2. | Compile a list of civil society expert in renewable energy and promote their work | Compilation of list of civil society representatives in the field of renewwable energy. | No list. | Number of expert by region,department and municipality. | Report | 2 years | 10,000 |  | MINEE, MINFOP, MINJEC, MINESUP, MINPMEESA, FEICOM and partners. |  |
|  | Overall Cost |  |  |  |  |  |  | 10,000 |  |  |
| PAA 3. | Introduce the green energy voucher and make tax and customs exemptions more acessible. | Identification of all authorised credit institution in Cameroon and lenders that can issue green voucher. | No Green voucher | Number of credit institution identified | Report | 3 years | 5,000 |  | MINEE, MINMID, MINRESI, Experts, etc. |  |
|  | Intégrating slora installation as a civic contribution. | Adopting a pilot programme to establish energy « prosumer ». | No pilot porjects | Number of energy packages | Report | 1 year | 10,000 |  | MINDUH,MINCAF,MINDDEVEL. |  |
|  | Overall Cost |  |  |  |  |  |  | 15,000 |  |  |
| PAA 4. | Accelerating reform in the electricity sector. | Drafting of implementing regulations covering all technicla aspects of revewable energy. | The reform began in 2011. | Numbers of implementing regulation available | Report | 5 years | 15,000 |  | MINDDEVEL,MINEE,MINEPDED, MINPMEESA |  |
|  | Overall Cost |  |  |  |  |  |  | 15,000 |  |  |