Original Research Article

Geochemical Assessment of the Heavy Metal Pollution in Superficial Sediments along Wadi as Sir Stream, Jordan

Abstract

The concentrations of Cu, Mn, Cd, Pb, and Zn in the superficial sediment samples at Wadi As-Sir area were determined in order to assess the level of pollution. Seventeen samples were collected through the period of April and July, 2019.  Nine samples from Wadi As-Sir, six samples from Wadi Al-Sheta, and two samples from Al-Kafrain Dam. The samples sediment pollution assessment was carried out using the contamination indicators such as Pollution Index (PI), pollution load index (PLI), and Geoaccumulation index (lgeo). Results of lgeo analyze of Wadi As-Sir stream sediments and Al-Kafrain Dam sediments showed be the concentrations of Mn, Cu, Cd and Pb are safe, while the concentration of Zn exceeded the average value. But results of lgeo analyze of Wadi Al-Sheta stream sediments showed the concentrations of Mn, Cu, and Pb are safe, while the concentration of Zn and Cd exceeded the average value. Results of PI analyze of Wadi As-Sir stream showed high concentrations of Zn and Cd, but results of PI analyze of Wadi Al-Sheta stream showed high concentrations of Zn, Mn and Cd, while results of PI analyze of Al-Kafrain Dam showed high concentrations of Zn only. The highest source of Zn, Mn and Cd originate mostly from anthropogenic sources such as irrigation return flow and the weathering of surrounding rock formations. The pollution load index (PLI) results indicate that there no pollution in the study area as low indices reported. Correlation coefficients analysis showed varying relation between the different parameters depending on the input sources of each metal. In order to protect the sediments from further contamination, monitoring and management of the heavy metal anthropogenic discharges is recommended.

Keywords: Sediments, Heavy Metals, Geochemical Assessment, Wadi As-Sir, Jordan.

1. Introduction

Environmental pollution is a problem in modern societies. Out of the various kinds of pollution, high contamination of aquatic systems with toxic heavy metals is the main concern provided. These elements are not biodegradable and that their eminent cultivation will affect food quality and safety. Heavy metals enter these aquatic systems mainly through natural inputs such as weathering and rock erosion and anthropogenic sources including human, industrial and agricultural activities, landfill runoff and sewage disposal (Cevik et al., 2009). Heavy metals discharged into aquatic systems have immobilized within stream sediments by main processes such as adsorption, flocculation and co-precipitation. Sediments in aquatic ecosystems therefore act as a buffer that can hold metals or transfer metals to the water column via various remobilization processes (Caccia et al., 2003; Pekey, 2006; Marchand et al., 2006). Several processes result in the association of heavy metals with solid phases, for example direct absorption by fine-grained inorganic clay particles; adsorption of hydrous ferric and magnetic oxides that might be associated with clays in turn adsorption or complication of inorganic or other organic compounds and subsequent accumulation as fresh solid phases (Gibbs, 1977). Numerous studies have shown that metal levels in sediments can be sensitive indicators of contaminants in aquatic systems such as by (Bellucci et al., 2002; Sekabira et al., 2003; Bloundi et al., 2009; Suthar et al., 2009; Mhamdi et al., 2010). Many approaches have applied in order to assess the severity of sediment contamination and to understand the natural and anthropogenic inputs in the river system. The metal assessment indices namely geo accumulation index (lgeo), pollution load index (PLI), Pollution Index (PI), Contamination Factor (CF), and sediment quality guidelines were often used to screen the potentiality for contaminants within sediments. The aims of these study assess the various sources of contamination to have (Cu, Zn, Cd, Mn, and Pb) to a prospect regarding the possible distribution and concentrations in sediments from the study area and prevention of this contamination. According to assessment will help in providing a better perspective about the amount and sources contamination in the study area, and which in turn will help in understanding the mechanisms driving and controlling. This contamination also aims to assess the various sources of contamination to have a prospect regarding the possible prevention of this contamination.

2.  Study Area

The Wadi as Sir Area is located to the west of Amman the capital city of Jordan in the central part of Jordan. The study area lies between 31°50' N and 31°57' N and 35°40' E and 35°50' E The study area includes three main streams and a dam. (Figure 1):

1- Wadi As-Sir Stream: it is a durable water flow. It stems from wadi As-sir region and passes along Al-Bassa area and Iraq Al-Amir. Agriculture abounds along the valley and also there are many parks for recreation in this area.

2- Wadi Al-Sheta stream: it is located in wadi As-sir district in Amman. This winding valley is a place that connects Iraq Al-Amir from the western side with Marj Al-Hamam region from eastern side. Agriculture can be noticed hugely by the side of the valley. It is a tourist attraction where park can exist such as Naba,a Al-Bahat and Wadi Al-Sheta park.

3- Al- Kafrain Dam: it is located in the middle Jordan – Valley in Al-Balqa Governorate it is used to store water for irrigation of crops.

The study area contains flat topography in some areas and steep slopes in other areas, the height of the study area ranges from 130 meters below sea level in the Kafreen dam area to 900 meters above sea level in Wadi Sir Area. (WAJ, 2002).

Figure 1: Satellite Image for the Study area shows the samples location.

3. Geological and structural Setting

The study area covered by superficial deposits for differentiated into Pleistocene sediments, alluvial, wadi sediments, and soil. Pleistocene Sediments consist of weakly indurated, poorly sorted gravels of Chert and limestone. Red-brown soil covers broad areas of the study area with variable thickness. The main outcrop formation covered the study area description and study by Shawabkeh, 2001; Diabat, and Abdel Ghafoor, 2004). These formations are following: Amman Silicified Limestone Formation. This formation consists mainly of dark brown to grey thick bedded Chert, silicified limestone, chalk, marl, siliceous coquina and chart phosphate, brecciated Chert and Tripoli. The formation was deposit in a shallow marine environment. Wadi Umm Ghudran Formation. The formation consists of massive, white buff-grey, hard, detrital chalk that contains fish teeth and shell fragments, thin layers of chart and quartzite. The formation was deposited in a pelagic environment, moderate to deep-water. The Wadi As Sir Formation, consists of massive limestone bedding hard buff dolomitic limestone with subsidiary marls and chart nodules common in the middle and upper part. The formation has been deposited in a fully marine environment. The Shuayb Formation consists of nodular limestone, marl, marly limestone, dolomitic limestone, coquina, fossiliferous limestone and nodular marly limestone. The formation was deposited in a subtidal marine environment, moderate to shallow. The Hummar Formation consists of grey limestone, dolomitic limestone and dolomite. The formation forms brown to grey-weathered cliffs. The formation was deposited in intertidal marine environment. The Fuhays Formation consists of grey-green marl, calcareous siltstone, thinly bedded nodular limestone and secondary gypsum. The formation was deposited in subtidal marine environment. The Na'ur Limestone Formation (Cenomanian) represents the first marine transgressive phase of the Tethys Ocean. The formation is up to 180 m thick. The basal part (Juhra Member) is 33 m and consists of fine- to medium-grained sandstone interbedded with siltstone, mudstone and marl. The upper part of the formation consists of shelly limestone, marl, dolomitic limestone and dolomite with chart nodules at the top. The formation was deposited in tidal to lagoon and open marine environments. The Kurnub Sandstones Group is exposed mainly in the western part of the study area and uncomfortably overlies either the Triassic or Jurassic rocks. This group ranges in thickness from 250 to 300 m and consists of white, medium- to coarse-grained, pebbly, cross-bedded Sandston. The upper part of this formation is varicolored, medium-grained, cross-bedded sandstone with burrows and plant remains. The Kurnub Sandstone Group was deposited in braided to meandering streams with shallow marine intervals. The Kurnub sandstone Group is uncomfortably overlain by a thick sequence of carbonate rocks represented by the Ajlun Group (Cenomanian-Turonian).

The study area is characterizing by a complex tectonic framework and high density of structural elements. Amman-Hallabat Structure is considered the majority distinctive structural feature in the study area. It crosses the entire study area and a lot of parts of the nearby Al Karama, Zarqa and Qasr Al Hallabat areas. This structure is mostly composed of dextral strike-slip fault accompanied by smaller conjugated antithetic and synthetic faults and sub-parallel folds (anticline and syncline). It strikes in general ENE-WSW in the study area, and curved to become in NE-SW trend in the adjacent sheet areas. The downthrown along the main fault of this structure is toward NNW, which was evidenced by the rock units on either side of the fault plan; Amman Formation exposed on the downthrown side against Wadi As Sir Formation in eastern Amman area, whereas Wadi As Sir Formation exposed against Na'ur Formation and the latter against Kurnub Sandstone Group in the western parts of the study area (Shawabkeh , 2001; Diabat,and Abdel Ghafoor 2004).

3. Methodology and analytical technique:

3.1. Samples Collection

Seventeen Stream sediment samples were collected along the main valley of Wadi Al Sir, Wadi Al-Sheta and Al-Kafrain dam. The samples collected at depths of 20 cm during (April to July 2019). The samples collected in a manner to cover the whole Superficial Sediments in the study area (Figure 1). The sample location in the main stream for wadi and in Al-Kafrain dam. Air dried the samples and put in stainless steel sieve to remove very large particles to have a homogenous and fine grin size. Afterward, samples transferred to labeled, pre-washed, polyethylene bags well transported to the laboratory for further sample preparation and analysis. The samples were prepared and then dried in the oven at 55°C for 2 hours. All dried samples were sieved; particles <63 μm (0.05-mm) in size were chosen to be analyzed as this size is proven to be the best size for analysis for arid and semi-arid regions (Saffarini and Lahwani, 1992).

3.2. Extraction

The extraction method was used by Ammonium-Acetate-EDTA to determine the heavy metal contents This method of extraction used for the determination of major as well as traces elements Cu, Cd, Zn, Mn and Pb. Extraction solution was prepared through adding 0.5M (Molarity) NH4Ac 0.5M+ HAc +0.02M EDTA (pH 4.65). Then, 38.5gm of NH4Ac dissolved in 500ml H2O + 25ml Acetic Acid and adding 5.845gm EDTA and then the volume raised to one litter with distilled water. After that, 20 gm. air-dried soil samples placed in a 300 ml Erlenmeyer flask, with adding 100 ml extracting solution. Then have shaken mechanically for 30 minutes. The suspension materials were filtered after shaking and the filtrate was collected in a polyethylene flask (Lakanen and Ervio, 1971). The concentrations of the trace elements for Cu, Cd, Zn, Mn and Pb were determined by using inductively coupled plasma optical emission spectrometer (ICP-OES) at water, Environment and Arid Regions Research Center, at Al al-Bayt University. Table1 shows the chemical analysis of the samples study.

3.3. Percent of Total Organic Matter (TOM)

The Organic Matter (TOM) determine by using equation (Dean 1974). A mass of sediment is dried at 105°C and subsequently heated for 2hour at 550°C. the Procedure was used:

1. Clean and dried crucibles at 105°C for 2 hours have been weighted and labeled.

2. 2g of sample placed in the crucible then sample and crucible weighted.

3. Crucibles placed with sample in muffle furnace at 550°C for two hours.

4. The crucible and samples weighted after heating.

5. The amount of (TOM) calculated by subtracted the weight in step (4) from the weight in step (2). The percent of (TOM) calculated by the following equation (Dean, 1974) (Table 1):

% (TOM) =weight sample before ashing-weight sample after ashingweight sample before ashing*100%. ….  Equation 1

3.4. Measurement pH

PH meter measurement for all the samples by using these Procedure:

1. Weigh 2 grams of the sediment sample, combine it with 20 milliliters of distilled water, mix them together for 5 minutes, and Let the sample calm down.

2. Measure the pH value of the sediments by the pH device (Table 1).

Table 1: Concentrations of trace Metals ppm for Cu, Cd, Zn, Mn, Pb, pH and TOM for the Study Samples.

| Sample No. | Cu | Cd | Zn | Mn | Pb | PH value | % (TOM) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Wadi As-Sir |  |  |  |  |  |  |  |
| 1 | 0.81 | 0.11 | 1.96 | 5.3 | 0.46 | 7.96 | 4.00 |
| 2 | 0.86 | 0.13 | 2.41 | 5.4 | 0.46 | 8.35 | 4.50 |
| 3 | 0.61 | 0.10 | 2.03 | 5.7 | 0.35 | 8.72 | 4.00 |
| 4 | 0.90 | 0.12 | 2.50 | 6.3 | 0.47 | 8.35 | 4.50 |
| 5 | 0.61 | 0.13 | 2.02 | 3.6 | 0.40 | 8.49 | 6.00 |
| 6 | 0.29 | 0.09 | 0.87 | 3.7 | 0.32 | 8.88 | 5.00 |
| 7 | 0.43 | 0.09 | 1.68 | 4.4 | 0.29 | 8.94 | 4.50 |
| 8 | 0.42 | 0.10 | 1.98 | 3.7 | 0.32 | 8.58 | 4.00 |
| 9 | 0.43 | 0.10 | 1.86 | 3.6 | 0.30 | 8.64 | 4.00 |
| Average | 0.59 | 0.10 | 1.92 | 4.26 | 0.374 | 8.54 | 4.50 |
| Wadi Al Sheta |  |  |  |  |  |  |  |
| 10 | 0.71 | 0.25 | 1.40 | 7.2 | 0.33 | 8.65 | 7.00 |
| 11 | 0.36 | 0.04 | 0.73 | 5.5 | 0.19 | 8.74 | 4.00 |
| 12 | 1.09 | 0.21 | 1.79 | 12.2 | 0.35 | 8.64 | 7.50 |
| 13 | 0.44 | 0.04 | 0.83 | 4.4 | 0.17 | 8.87 | 4.50 |
| 14 | 0.70 | 0.23 | 1.18 | 10.4 | 0.28 | 8.75 | 7.00 |
| 15 | 0.42 | 0.04 | 0.76 | 4.5 | 0.16 | 8.96 | 4.00 |
| Average | 0.62 | 0.13 | 1.11 | 7.36 | 0.24 | 8.76 | 4.66 |
| Al Kafrain Dam |  |  |  |  |  |  |  |
| 16 | 0.07 | 0.02 | 0.37 | 7.0 | 0.49 | 8.94 | 4.00 |
| 17 | 0.71 | 0.03 | 1.82 | 5.4 | 1.29 | 8.86 | 4.50 |
| Average | 0.388 | 0.025 | 1.094 | 6.180 | 0.891 | 8.90 | 4.25 |

4. Assessment of Contamination Level

The measure level of metals contamination in the superficial sediments for the area study, was used calculation methods of the Geoaccumulation index (Igeo), Pollution load index (PLI) and Pollution Index (PI) to assess the metals contamination (Yang et al., 2011). The Geoaccumulation index (Igeo was used by Muller, (1981) and Wei et al. (2009) to assess heavy metal pollution. The Igeo was used to assess metal contamination in urban soils by comparing current and pre-industrial values (Wei et al. 2009; Al-Khashman, 2013). In this study, the geoaccumulation index (Igeo) was used to determine the extent of heavy metal pollution in Wadi As Sir Superficial sediments. The following equations for Rath et al., 2005 was used in to the calculation:

Igeo = log2 (Cn/1.5Bn), -------------------------------equation1

Where Cn is the concentration of the element ‘n’, and Bn is the geochemical background value of the element n, and 1.5 is the background matrix correction factor due to lithogenic effects. The geochemical background value in average shale is used to calculate Igeo (Turekian and Wedepohl, 1961). The geo-accumulation index (Igeo) scale consists of seven grades from 0 to 6, ranging from unpolluted to highly polluted (Table 2) (Muller, 1981,  Rath et al., 2005, Faiz et al., 2009). The Geoaccumulation index value class designation of sediment quality is as follows: Igeo < 0 unpolluted environments >5 extremely polluted.

The pollution index (PI) and Pollution load index (PLI) were used to assess heavy metal pollution in the soil, Superficial sediments and dust. The pollution index (PI) was defined by the following equation:

PI = Cn /Bn ------------------------------------------equations 2

where Cn and Bn are the measured and background concentrations of metal n, respectively, in the Superficial sediment samples. PI is classified by (Faiz et al., 2009) as follows:

PI ≤ 1 indicates a low level of pollution;

1< PI ≤ 3 indicates a middle level of pollution, and

PI > 3 a high level of pollution.

The pollution Load Index (PLI) for each sample was calculated following the method by Tomilson et al., (1980) and Soares et al., (1999). It was defined by the following equation:

PLI=nCF1+CF2+CF3+…=CFn ------equation3

Where; n is the number of metals and CF is the contamination factor. The contamination factor (CF) can be calculated from the following relation:

(CF) = Metal concentration in the sediments /Background value of the metal……Equation 4

According to Harikumar et al.2009, and Seshan et al., (2010) the PLI value

(PLI ≤ 1) indicates a low level of pollution, while

1< PLI ≤ 2 indicates a middle level of pollution and

PLI >2 high level pollution.

Table 2: The degree of metal pollution in terms of seven enrichment classes for (Igeo)

| Igeo Value | Igeo Class | Designation of sediment quality |
| --- | --- | --- |
| >5 | 6 | Extremely polluted |
| 4-5 | 5 | Strongly to extremely polluted |
| 3-4 | 4 | Strongly polluted |
| 2-3 | 3 | Moderately to Strongly polluted |
| 1-2 | 2 | Moderately polluted |
| 0-1 | 1 | Un polluted to moderately polluted |
| <0 | 0 | Un polluted |

5. Results and Discussion

5.1. Physicochemical Properties of the Superficial Sediments

The chemical concentrations of the heavy metals in the Superficial sediments of the area study (Wadi As-Sir, Wadi Al Sheta and Al Kafrain Dam) reflected to the strength of anthropogenic, natural sources and PH values (Table 1). They have variable chemical concentration of heavy metal distributions along the region of studied sample Figure 2. The high levels of Mn and Zn were found along the superficial sediments of Wadi As-Sir and Wadi Al-Sheta and Al Kafrain dam from upstream to downstream, but low concentrations of Cu, Cd and Pb were found along the valley. The average concentrations of Cu, Cd, Zn, Mn and Pb in Wadi As-Sir sediments are (0.596, 0.107, 1.923, 4.262 and 0.374). The average concentrations of Cu, Cd, Zn, Mn and Pb in Wadi Al Sheta sediments are (0.621, 0.133, 1.115, 7.363 and 0.244). The mean concentrations of Cu, Cd, Zn, Mn and Pb in Al Kafrain Dam sediments are (0.388, 0.025, 1.094, 6.180 and 0.891) respectively and have been compared with unpolluted sediments and soils (Bowen, 1979). The high concentrations of heavy metals in the sediments of Wadi As-Sir were possibly because to the anthropogenic sources also human and agricultural activities near the wadi surface sediments’ site (Calace et al., 2005). The sources include fertilizers and pesticides used in agricultural activities and discharges of sewage wastewater and surface water drainage near areas of human activities along the Wadi As-Sir area.

The pH values show a relatively equal distribution in the Study Area Figure 3. The pH-values range between7.96 and 8.96 (Table 1). The high PH value effected to more alkaline conditions for all the sediment samples. The concentrations of metals in the sediments can vary highly depending on the organic matter content and pH-values. The PH values influence the sediment composition, and which turn affects the heavy metal ions mobility and attractive to distribution in the sediment (Odat and Alshammari, 2011). The pH values show a relatively equal distribution in the investigated area. The average value of PH 8.66, suggesting alkaline-prevailing conditions for all the sediment samples. The concentrations of metals in the sediments can vary greatly depending on the organic matter content and pH-values. Usually, pH-values influence the sediment composition, which in turn affects the heavy metal mobility and distribution in the sediment samples (Odat and Alshammari, 2011).

The percentage of organic matter (OM%) in the Superficial Sediments of Wadi As-Sir range between 4.0% to 7.5% with an average of 4.88 % (Table 1). The amount of organic matter in Wadi As-Sir sediments is linked to the organic material of plant remains at different stages of decomposition, deposited adjacent to the Wadi location Figure 4. Cells and tissues of plant organisms and substances from plant roots, and soil microbes are considered as additional sources (Figure 3).

Figure 2:  Distribution for the Heavy Metal Concentrations of the Studied Sample for Wadi As-Sir area.

Figure 3: pH Values Distribution of the sediment for Studied Sample

Figure 4: Organic Matter (TOM) Distribution for Superficial Sediments of the Studied Samples

The Pearson’s correlation coefficients between the contents of Cu, Zn, Cd, Mn and Pb in the Superficial Sediments of Wadi As-Sir area listed in Table 2. Significant correlation of Cu with Zn (r = 0.701) and moderated correlation of Cu with Cd (r = 0.622), Mn with Cd (r = 0.614) and Mn with Cu (r = 0.527), The results revealed that metal concentrations rise abnormally due to the anthropogenic sources of contamination which have been identified to be the human, industrial, and agricultural activities such as sewage effluent, waste combustion, fertilizers and pesticides (Ennouri et al. 2010). The significant correlations were clearly observed for Pb with Cu, Zn, Cd and Mn, Mn with Zn ; Zn with Cd. These results indicate that the heavy metals analyzed are not interrelated, and there is no relationship between the variables (Bany Yaseen and Al-Hawari, 2015). Moreover, these metals have different anthropogenic and natural sources in the sediments of Wadi As-Sir area (Habes and Nigem, 2006, Bany Yaseen and Elaimat, 2016).

Table 2: Correlation coefficients of the Heavy Metals for the Studied Samples

|  | Cu | Cd | Zn | Mn | Pb |
| --- | --- | --- | --- | --- | --- |
| Cu | 1 |  |  |  |  |
| Cd | 0.622 | 1 |  |  |  |
| Zn | 0.701 | 0.345 | 1 |  |  |
| Mn | 0.527 | 0.614 | -0.045 | 1 |  |
| Pb | 0.238 | -0.193 | 0.306 | 0.013 | 1 |

5.2. Geoaccumulation Index (Igeo)

The Igeo distribution in the study samples are shows in Table 3, the percent of Igeo differs according to metal type and location. Samples from 1 to 9 indicate to the sediments of Wadi As-Sir stream, Mn, Cd, Pb and Cu remain in grade < 0 (unpolluted) in Wadi As-Sir stream. While Zn to strongly polluted; depending on the sample position and the concentration of the contribution elements. The highest grade of Igeo was attributed to Zn because it exceeds 4 in some places, showing that the sediment samples are extremely polluted. This indicates that Wadi As Sir sediments have background concentrations for Zn; These elements are practically altered by anthropogenic influences. These hazardous metals can be derived from industrial waste and gasoline additives used in the automotive and industrial sectors (Mwamburi 2003). Furthermore, they may be derived from corrosion agricultural activities in the wadi. While samples from 10 to 15 indicate to the sediments of Wadi Al-Sheta stream, the Igeo values indicate that the sediments of Wadi Al-Sheta are unpolluted with Cu, Mn, and Pb remain in grade zero in all stations (Figure 5), while Zn and Cd moderately to strongly polluted, the highest grade of Igeo was attributed to Zn because it exceeds 3 in some places, showing that the sediment samples are extremely polluted. This indicates that Wadi Al-Sheta sediments have background concentrations for Zn and Cd; these elements are practically altered by anthropogenic influences. These hazardous metals can be derived from industrial waste and gasoline additives used in the automotive and industrial sectors (Mwamburi 2003; Bany Yaseen and Harahsha, 2024)

Table 3: The Igeo Values for Heavy Metal in the Studied Sample for Wadi As-Sir, Wadi Al Sheta and Wadi Al Kafrain dam.

| Geoaccumulation Index (Igeo) for Wadi As-Sir samples |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. of Samples | Cu | Cd | Zn | Mn | Pb |
| 1 | -5.525 | -0.418 | 3.781 | -1.221 | -6.02 |
| 2 | -5.444 | -0.133 | 4.081 | -1.195 | -6.024 |
| 3 | -5.934 | -0.499 | 3.835 | -1.11 | -6.433 |
| 4 | -5.374 | -0.341 | 4.131 | -0.968 | -6.011 |
| 5 | -5.953 | -0.222 | 3.822 | -1.754 | -6.232 |
| 6 | -7.024 | -0.773 | 2.606 | -1.719 | -6.537 |
| 7 | -6.443 | -0.676 | 3.56 | -1.496 | -6.682 |
| 8 | -6.466 | -0.555 | 3.797 | -1.723 | -6.537 |
| 9 | -6.456 | -0.629 | 3.103 | -1.792 | -6.653 |
| Average | -6.068 | -0.471 | 3.635 | -1.442 | -6.347 |
| Geoaccumulation Index (Igeo) for Wadi Al Sheta samples |  |  |  |  |  |
| 10 | -5.718 | 0.771 | 3.295 | -0.768 | -6.515 |
| 11 | -6.686 | -1.842 | 2.352 | -1.161 | -7.333 |
| 12 | -5.101 | 0.479 | 3.653 | -0.016 | -6.438 |
| 13 | -6.403 | -1.914 | 2.535 | -1.484 | -7.48 |
| 14 | -5.751 | 0.652 | 3.049 | -0.236 | -6.758 |
| 15 | -6.48 | -2.029 | 2.422 | -1.46 | -7.559 |
| Average | -6.023 | -0.647 | 2.884 | -0.854 | -7.013 |
| Geoaccumulation Index (Igeo) for the sediments in Wadi Al Kafrain dam |  |  |  |  |  |
| 16 | -9.107 | -2.807 | 1.376 | -0.821 | -5.927 |
| 17 | -5.726 | -2.341 | 3.674 | -1.187 | -4.539 |
| Average | -7.415 | -2.574 | 2.525 | -1.004 | -5.233 |

Figure 5: The Igeo Values Distribution for Heavy Metal in the Studied Sample

5.3. Pollution Index (PI)

The results of PI for the study samples shows in Table 4 and Figure 6. The PI values for all metals in Wadi As-Sir Superficial Sediments samples low level pollution (PI  1)  for  Cu, Mn, and Pb, where Cd are medium levels of pollution(1< PI  3) but Zn are high levels of pollution (PI > 3). The results of PI value of Wadi Al-Sir Superficial Sediments show Cu, Mn and Pb are safe, these elements are practically unchanged due to anthropogenic influences. While the PI value of Zn, and Cd exceeds the medium and high levels of pollution. Medium to high PI value of Cd and Zn may be due to anthropogenic sources. The current study confirmed that long-term use of phosphate fertilizer that contains a high level of Cd may lead to accumulation of Cd in the soil, and this may eventually increase plant Cd uptake (El-Shebiny et al., 1997; lone, et al.,2008; Dokmeci, 2020). The PI results for of the samples in wadi Al Sheta shows Cu, and Pb are safe for low levels of pollution (PI  1). These elements are practically unchanged due to anthropogenic influences. Cd, and Mn are medium levels of pollution (1< PI  3) but Zn are high levels of pollution (PI > 3). The elevated value PI of Cd, Mn and Zn may be due to anthropogenic sources for the study area (Abu-mayyaleh, 2025). The results of PI for Al Kafrain dam shows Cu, Mn, Cd and Pb are safe, low levels of pollution (PI  1). These elements are practically unchanged due to anthropogenic influences, but Zn are high levels of pollution (PI > 3), due to anthropogenic influences sources.

Table 4: The Pollution Index Values for Heavy Metal in the Studied Sample in Wadi As-Sir, Wadi Al Shta and Al Kafrain Dam.

| Pollution Index (PI) for Wadi-Sir Samples |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Sample No. | Cu | Cd | Zn | Mn | Pb |
| 1 | 0.032 | 1.122 | 20.620 | 0.643 | 0.023 |
| 2 | 0.034 | 1.367 | 25.400 | 0.654 | 0.023 |
| 3 | 0.024 | 1.061 | 21.410 | 0.694 | 0.017 |
| 4 | 0.036 | 1.183 | 26.290 | 0.766 | 0.023 |
| 5 | 0.024 | 1.285 | 21.220 | 0.444 | 0.019 |
| 6 | 0.011 | 0.877 | 9.136 | 0.455 | 0.016 |
| 7 | 0.017 | 0.938 | 17.690 | 0.531 | 0.014 |
| 8 | 0.016 | 1.020 | 20.860 | 0.454 | 0.016 |
| 9 | 0.017 | 0.969 | 19.530 | 0.432 | 0.014 |
| Average | 0.023 | 1.091 | 20.030 | 0.563 | 0.018 |
| Pollution Index (PI) for Wadi Al Shta samples |  |  |  |  |  |
| 10 | Cu | Cd | Zn | Mn | Pb |
| 11 | 0.028 | 2.651 | 14.720 | 0.880 | 0.016 |
| 12 | 0.014 | 0.418 | 7.663 | 0.670 | 0.009 |
| 13 | 0.043 | 2.091 | 18.870 | 1.482 | 0.017 |
| 14 | 0.027 | 2.357 | 12.42 | 1.273 | 0.013 |
| 15 | 0.017 | 0.367 | 8.042 | 0.544 | 0.007 |
| Average | 0.024 | 1.38 | 11.73 | 0.897 | 0.011 |
| Pollution Index (PI) for Al Kafrain Dam samples |  |  |  |  |  |
| 16 | 0.002 | 0.214 | 3.894 | 0.848 | 0.024 |
| 17 | 0.028 | 0.295 | 19.14 | 0.658 | 0.064 |
| Average | 0.015 | 0.254 | 11.51 | 0.753 | 0.044 |

Figure 6: The Pollution Index (PI) Values Distribution for Heavy Metal in the Studied Sample

5.4. Pollution Load Index (PLI)

The PLI distribution in the study samples are shows in  Figure 7 and Table 5, the results value of PLI indicate in to no pollution (PLI < 1) in all the study samples for the area study include (Wadi As-Sir stream, Wadi Al Sheta and Al Kafrain dam).

Figure 7: The PLI Values Distribution for Heavy Metal in the Studied Sample

Table 5: Contamination factor (CF) and pollution Load Index (PLI) for the study samples

| (CF) and (PLI) for Heavy Metal in Wadi As-Sir area |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Contamination Factor (CF) |  |  |  |  |  | PLI |
| No. of samples | Cu | Cd | Zn | Mn | Pb | PLI |
| 1 | 0.032 | 1.122 | 20.62 | 0.643 | 0.023 | 0.405 |
| 2 | 0.034 | 1.367 | 25.4 | 0.654 | 0.023 | 0.446 |
| 3 | 0.024 | 1.061 | 21.41 | 0.694 | 0.017 | 0.365 |
| 4 | 0.036 | 1.183 | 26.29 | 0.766 | 0.023 | 0.456 |
| 5 | 0.024 | 1.285 | 21.22 | 0.444 | 0.019 | 0.356 |
| 6 | 0.011 | 0.877 | 9.136 | 0.455 | 0.016 | 0.23 |
| 7 | 0.017 | 0.938 | 17.69 | 0.531 | 0.014 | 0.293 |
| 8 | 0.016 | 1.02 | 20.86 | 0.454 | 0.016 | 0.301 |
| 9 | 0.017 | 0.969 | 19.53 | 0.432 | 0.014 | 0.29 |
| Average | 0.023 | 1.091 | 20.03 | 0.563 | 0.018 | 0.394 |
| (CF) and (PLI) for Heavy Metal in Wadi Al Sheta area |  |  |  |  |  |  |
| Contamination Factor (CF) |  |  |  |  |  | PLI |
| No. of samples | Cu | Cd | Zn | Mn | Pb | PLI |
| 10 | 0.028 | 2.651 | 14.72 | 0.88 | 0.016 | 0.43 |
| 11 | 0.014 | 0.418 | 7.663 | 0.67 | 0.009 | 0.194 |
| 12 | 0.043 | 2.091 | 18.87 | 1.482 | 0.017 | 0.534 |
| 13 | 0.018 | 0.397 | 8.694 | 0.535 | 0.008 | 0.194 |
| 14 | 0.027 | 2.357 | 12.42 | 1.273 | 0.013 | 0.425 |
| 15 | 0.017 | 0.367 | 8.042 | 0.544 | 0.007 | 0.184 |
| Average | 0.024 | 1.38 | 11.73 | 0.897 | 0.011 | 0.334 |
| (CF) and (PLI) for Heavy Metal Al Kafrain dam |  |  |  |  |  |  |
| Contamination Factor (CF) |  |  |  |  |  | PLI |
| No. of samples | Cu | Cd | Zn | Mn | Pb | PLI |
| 16 | 0.002 | 0.214 | 3.894 | 0.848 | 0.024 | 0.128 |
| 17 | 0.028 | 0.295 | 19.14 | 0.658 | 0.064 | 0.367 |
| Average | 0.015 | 0.254 | 11.51 | 0.753 | 0.044 | 0.247 |

6. Conclusions

The main conclusions of this study can be summarized as follows:

- The chemical concentrations of the heavy metals in the Superficial sediments of the area study shows to the strength of anthropogenic and natural sources within PH values The high levels of Mn and Zn were found along the superficial sediments of Wadi As-Sir and Wadi Al-Sheta and Al Kafrain dam from upstream to downstream, but low concentrations of Cu, Cd and Pb were found along the valley.

-  The high PH value effected to more alkaline conditions for all the sediment samples. The concentrations of metals in the sediments can vary highly depending on the organic matter content and pH-values. The PH values influence the sediment composition, and which turn affects the heavy metal ions mobility and attractive to distribution in the sediment. The concentrations of metals in the sediments can vary greatly depending on the organic matter content and pH-values.

- Pearson’s correlation coefficients analysis of concentrations among Cu, Zn, Cd, Mn, and Pb. shows varying correlations among them, which indicates that these metals have complex geochemical behaviors.

-The results of Igeo value of Wadi Al-Sir sediments have shown that the concentrations of Mn, Cu, Cd and Pb are safe and that these elements are practically unchanged due to anthropogenic influences, while the concentrations of Zn exceed the average value. The elevated concentrations of Zn may because of anthropogenic sources, for example phosphate fertilizers and pesticides used in agricultural lands in the wadi and agricultural use of sewage sludge and seepage of domestic wastewater from cesspits in nearby villages to the wadi and natural weathering of surrounding geological formations.

-The results of Igeo analyzes of Wadi Al-Sheta sediments have shown that the concentrations of Mn, Cu, and Pb are safe and that these elements are practically unchanged due to anthropogenic influences, while the concentrations of Zn and Cd exceed the average value. The elevated concentrations of Zn and Cd may because of anthropogenic sources.

-The results of Igeo analyzes of Al Kafrain Dam have shown that the concentrations of Mn, Cd, Cu, and Pb are safe and that these elements are practically unchanged due to anthropogenic influences, while the concentrations of Zn exceed the average value. The elevated concentrations of Zn may because of anthropogenic sources.

-The PLI analysis proved that Wadi Al-Sir area is moving towards a probable environmental pollution with regard to heavy metals (Zn and Cd) if experts did not control these external sources.

-The results of PI analyze of Wadi Al-Sir sediments have shown that Cu, Mn and Pb concentrations are safe and that these elements are practically unchanged due to anthropogenic influences, while the concentration of Zn, and Cd exceeds the medium and high levels of pollution. Elevated concentrations of Cd and Zn may be due to anthropogenic sources. The current study confirmed that long-term use of phosphate fertilizer that contains a high level of Cd may lead to accumulation of Cd in the soil, and this may eventually increase plant Cd uptake.

-The results of PI analyze of Wadi Al-Sheta sediments have shown that Cu, and Pb concentrations are safe and that these elements are practically unchanged due to anthropogenic influences, while the concentration of Zn, Mn and Cd exceeds the medium and high levels of pollution. Elevated concentrations of Cd, Mn and Zn may be due to anthropogenic sources.

-The results of PI analyze of Al Kafrain sediments have shown that Cu, Mn, Cd and Pb concentrations are safe and that these elements are practically unchanged due to anthropogenic influences, while the concentration of Zn, exceeds the high levels of pollution.

-The soil characteristics, such as clay content of the soil organic matter and calcium carbonate and degrees of salinity and acidity with a key role on the content of the soil from cadmium. Natural weathering of surrounding rock formations can act as another source of heavy metals in the sediments, increase the proportion of lime in the soil with a high temperature help to increase the adsorption cadmium in calcareous soils.

-The chief contamination sources of Zn and Cu in some samples are agricultural use of sewage sludge and the use of agro-chemicals such as phosphate fertilizers and pesticides, moreover, the proliferation of septic tanks in residential communities bordering put him in addition to its cargo tanks seepage in the neighboring valleys.

-Variation of the concentration of Mn in the study area was because of the irrigation of land by industrial wastewater and other agronomic practices and the high value of organic matter. The major anthropogenic sources of Mn include municipal wastewater discharges, sewage sludge, Industries in the Wadi Al- Sir area.

6. References

- Abu-mayyaleh S. A. (2025). Geochemical Assessment of the Level Toxic Heavy Metals Pollution for the Surficial Deposits in Birin area, Jordan., thesis Deanship of Graduate Studies, AL al-Bayt University.

- Al-Khashman, O.A. (2013). Assessment of heavy metals contamination in deposited street dusts in different urbanized areas in the city of Ma’an, Jordan. Environ Earth Sci. 70: 2603- 2612

- Bany Yaseen and Harahsha,( 2024). Assessment of Pollution by Heavy Metals in Surface Sediments at Wadi Al-Wala, Jordan. Asian Journal of Geological Research, Volume 7, Issue 2, Page 120-135,

- Bany Yaseen, I. and Al Elaimat.A., (2016). Assessment of the Heavy Metal Pollution in the Surface Sediments along Upstream at Wadi Al Arab, Jordan, Journal of Natural Sciences Research  Vol.6, No.20.

- Bany Yaseen, I. and Al-Hawari,Z., (2015). Assessment of Heavy Metal Pollution of Surface Sediments in Wadi Shu'ayb, Jordan) Jordan Journal of Civil Engineering, Vol. 9, No. 3.

- Bellucci, L.G., M. Frignani, D. Paolucci and M. Ravanelli, (2002). Distribution of heavy metals in sediments of the Venice Lagoon: The role of the industrial area. Sci. Total Environ. 295- 3549.

-- Bloundi, M.K., Duplay, J., Quaranta, G. (2009). Quaranta heavy metal contamination of coastal lagoon sediments by anthropogenic activities: The case of Nador (East Morocco), Environ. Geol., 56: 833-843.

- Bowen, H.J.M. (1979). Environmental chemistry of the elements. Academic Press, New York.

- Caccia, V.G., F.J.Millero and A. Palanques, (2003), The distribution of trace metals in Florida Bay sediment. Mar. Pollut. Bull., 46(11): 1420-1433.

- Calace, N., Ciardullo, S., Petronio, B.M., Pietrantonio, M., Abbondanzi, F., Campisi, T., Cardellicchio, N. (2005). Influence of chemical parameters (heavy metals, organic matter, sulphur and nitrogen) on toxicity of sediments from the Mar Piccolo (Taranto, Ionian Sea, Italy). Microchem J., 79 (1–2): 243–248.

- Cevik, F., M.Z.L. Göksu, O.B. Derici and Ö. Findik, (2009). An assessment of metal pollution in surface sediments of Seyhan dam by using enrichment factor , Geoaccumulation index and statistical analyses. Environ. Monit.Assess, 152: 309317.

-  Dean, W.E.J., 91974). Determination of carbonate and organic matter in calcareous sediments and sedimentary rocks by loss on ignition: Comparison with other methods. Journal of Sedimentary Petrology, 44(1), pp.242—248.

- Diabat, A., and Abdel Ghafoor ,M., (2004) Geological map of Amman ,sheet no., (3153-1) scale:1/50,000. Natural Resources Authority, Geological Mapping Division, Geology Directorate. Amman, Jordan.

- Dökmeci, A. H. (2020). Environmental impacts of heavy metals and their bioremediation. In Heavy metals-their environmental impacts and mitigation. Intech Open.

- Ennouri R, Chouba L, Magni P, Kraiem MM, (2010): Spatial distribution of trace metals (Cd, Pb, Hg, Cu, Zn, Feand Mn) and oligo-elements (Mg, Ca, Na and K) in surface sediments of the Gulf of Tunis (NorthernTunisia). Environ Monit Assess., 163:229–239.

- El-Shebiny, G.M., E.A. Mashaly and M. M.El-Garawany. (1997). Kinetics of cadmium sorption in soils. Egypt. J. Appl. Sci. 12 :( 12) 288-303.

- Faiz, Y., Tufail, M., Javed, M.T., Chaudhry, M.M. (2009). Road dust pollution of Cd, Cu, Ni, Pb and Zn along Islamabad Expressway, Pakistan. Micro chemical Journal, 92(2): 186-192.

-  Gibbs, R.J., (1977), Transport phases of transition metals -in the Amazon and Yukon Rivers. Geol. Soc. Am. Bull. 88, 829-843.

- Habes, G., and Nigem, Y. (2006): “Assessing Mn, Fe, Cu, Zn and Cd pollution in bottom sediments of Wadi Al- Arab Dam, Jordan". Chemosphere, 65, 2114-2121.

-   Harikumar, P., Nasir, U. & Rahman, M.M., (2009). Distribution of heavy metals in the core sediments of a tropical wetland system. Int. J. Environ. Sci. Tech, 6(2), pp.225 232.

-  Lakanen, E., Ervio, R., (1971). A comparison of eight extractants for " the determination of plant available micronutrients on soil. Acta Agric. Fenn. 123, 223—232.

- Lone, M. I., He, Z. L., Stoffella, P. J., & Yang, X. E. (2008). Phytoremediation of heavy metal polluted soils and water: progresses and perspectives. Journal of Zhejiang University Science B, 9(3), 210-220.

- Marchand, C., E. Lalliet Verges, F. Baltzer, P. Alberic, D. Cossa and P. Baillif, (2006). Heavy metals distribution in mangrove sediments along the mobile coastline of French Guiana. Mar. Chem., 98: 1-17.

- Muller G. (1981). Index of Geoaccumulation in sediments of the Rhine River .Geojournal, 2(3): 108-118.

- Mwamburi J. (2003). Metal Sources and Distribution in Rivers within the Lake Victoria (Kenya) Catchment Area . Lake Victoria Fisheries: Status, biodiversity and management. Aquatic Ecosystem Health and Management Society.

- Odat, S., and Alshammari, A. (2011). Spacial Distribution of Soil Pollution along the Main Highways in Hail City, Saudi Arabia, Jordan Journal of Civil Engineering, 5(2): 163-172.

- Pekey, H. (2006). Heavy Metal Pollution Assessment in Sediments of the Izmit Bay, Turkey. Environ. Monit. Assess., 123: 219-231.

- Rath P, Panda UC, Bhatta D, Sahoo BN,  (2005). Environmental Quantification of Heavy Metals in the Sediments of the Brahmani and NandiraRivers , Orissa. J GeolSocInd, 65: 487-492.

- Saffarini, G.A., and Lahawani, Y. (1992), Multivariate statistical techniques in geochemical exploration applied to Wadi sediments data from an arid region: Wadi SW Jordan, J. Afr. Earth Sci. 14(3), 417-427.

- Sekabira, K., H. OryemOriga, T.A. Basamba, G. Mutumba, E. Singh, A.K., S.I. Hasnain and D.K. Banerjee, (2003). Grain size and geochemical portioning of heavy metals in sediments of the Damodar River-a tributary of the lower Ganga India. Environ. Geol.

- Seshan, B. R. R.; Natesan, U.; Deepthi, K., (2010). Geochemical and statistical approach for evaluation of heavy metal pollution in core sediments in southeast coast of India. Int. J. Environ. Sci. Tech., 7 (2), 291-306 (16 pages).

- Shawabkeh, KH., (2001). Geological map of Al-Karama (3153-1V), scale:1/50,000. Natural Resources Authority, Geological Mapping Division, Geology Directorate. Amman, Jordan.

- Soares H MVM ,Rui A R Boaventura, Adelio A S C Machado, and Joaquim C G Esteves da Silva, 1999. Sediments as monitors of heavy metal contamination in the Ave river basin (Portugal): Multivariate analysis of data. Environmental Pollution, 105(3), pp.311-323.

- Suthar, S., A.K. Nema, M. Chabukdhara and S.K. Gupta, 2009. Assessment of metals in water and sediments of Hindon River, India: Impact of industrial and urban discharges. J. Hazard. Mater, 171(1-3): 1088-1095.

- Tomlinson, D.L. Wilson, J.G Harris, C.R. and Jeffney, D.W. 1980, Problems in the assessment of heavy metal levels in estuaries and the formation of a pollution index, Helgol,Wiss. Meeresunters, Vol.33, pp. 566-572.

- Turekian KK, Wedepohl KH, 1961. Distribution of the elements in some major units of the earth's crust . Bulletin of Geological Society of America 1961 ; 72: 175—92.

-Wei, B., Jiang, F., Li, X., Sh, Mu. (2009). Spatial distribution and contamination assessment of heavy metals in urban street dusts from Urumqi, NW China. Microchem J., 93:147–152.

- Water Authority of Jordan (WAJ) , 2002. Groundwater Control BY Law No. 85/2002.

- Yang, Z., Lu, W., Long, Y., Bao, X., Yang Q. (2011). Assessment of heavy metals contamination in urban topsoil from Changchun City, China. J Geochem Explor 108: 27–38.