Assessment of the Level of Antibiotics and Biocide Residues in Atavu River Stretch Passing through Akpugo and Amodu, Nigeria

Abstract

The presence of antibiotics and biocidal agents in aquatic environments raises significant concerns due to their potential impacts on water quality and human health. This study aimed to detect residues of various antibiotics and biocides, in the Atavu River. Samples were collected from two distinct communities, Akpugo and Amodu, along the river stretch, as well as from a spring water source located on the same river site at Akpugo, Enugu. Fourteen (14) antimicrobial (antibiotics and biocides) from the rivers randomly collected from the head, mid, and tail regions and spring water were analyzed using a Gas Chromatography Flame Ionization Detector (GC-FID). Different levels of antibiotics and biocides were recorded in the head, mid and tail regions of the river within both communities. Notably, concentration of tetracycline among all detected biocides in the river samples from Akpugo, and Amodu ranges from 0.10-0.19ug/ml and 0.50 -0.65ug/ml, respectively. Other biocides which include penicillin, dicloxacillin, oxacillin, Amoxacillin, Ampicillin, Avilamycin, Gentamycin, Colistin, Lincomycin and Tylosin gave a concentration range of 0.01 – 0.18ug/ml. The pesticide aldrin (0.01ug/ml) and an antimalarial parent compound, quinoline (0.03ug/ml) was detected in mid region of Akpugo and head and downstream region of Amodu rivers respectively. Lincomycin and quinoline (0.01ug/ml) were detected in the spring water from Amodu respectively. The pervasive presence of antibiotics and biocidal residues in both the river and spring water is a considerable concern, as the river serves as a crucial source of drinking water and is utilized for agricultural activities, while the spring water is the primary source of potable water for the community. This persistent exposure to antibiotics and  biocide-laden waters suggests a potential for adverse health effects and the development of resistance in microbial populations, highlighting an urgent need for intervention.

KEY WORDS: Antibiotics, Biocide, Atavu River, Water Quality and GC-FID

INTRODUCTION

Water bodies worldwide, including rivers, lakes, and streams, are increasingly threatened by pollution from various anthropogenic activities. Among these, the contamination of water resources by biocides, such as pesticides, fungicides, and antimicrobial agents, has emerged as a significant environmental and public health concern. Antibiotics and biocides are widely used in agriculture, industry, and domestic settings to control pests, diseases, and microbial growth. However, their indiscriminate use and improper disposal have led to their accumulation in the environment, particularly in water bodies, through runoff, leaching, and effluent discharge (Acharya et al., 2025; Akhta et al., 2021).

The Atavu River stretch passing through Akpugo and Amodu in Nigeria is a vital water resource supporting local communities, agriculture, and ecosystems. Despite its importance, this river stretch is potentially vulnerable to pollution from agricultural runoff, industrial effluents, and domestic waste, which may contain residues of biocides. The presence of these residues in water bodies can have detrimental effects on aquatic life, human health, and the overall ecosystem balance (Jimenez  et al., 2023; Kübler-Ross, 2014).

Given the critical role of water quality in sustaining life and the environment, it is essential to monitor and assess the levels of biocide residues in water resources. Recent studies emphasize the importance of understanding the environmental and health implications of biocide use (Tomasello et al., 2003). This study aims to evaluate the extent of antibiotics and biocide contamination in the Atavu River stretch and a spring water, providing valuable insights into the environmental and health implications of biocide use in the region.

METHODOLOGY

Study Area and Sampling Design

Water samples were collected in May 2024 during a single sampling campaign conducted in the wet season, a period characteristic of tropical regions. The first part of the experiment was carried out on the water samples gotten from river atavu located at Akpugo, while the second phase consists of the water samples gotten from the same river atavu but located at Amodu both in Nkanu west LGA, Enugu State, Nigeria . This season was intentionally chosen because it coincides with increased water flow, agricultural runoff, and variations in antibiotic consumption patterns due to changes in disease incidence and tourism activity.

The study covered four sampling points in total — three from the river and one from a spring water source used for drinking. The river sampling points included: Upstream: representing areas with minimal anthropogenic influence, located away from residential or agricultural activities, Midstream: representing areas moderately influenced by human and agricultural activities, and Downstream: representing areas of higher anthropogenic pressure, located near settlements and livestock farms. The spring water sample was obtained from a local drinking-water source situated within the same hydrological catchment.

These sites were selected to reflect a gradient of anthropogenic impact, ranging from low (upstream) to high (downstream), enabling comparison of antibiotic concentrations under varying levels of human, agricultural, and livestock pressure.

Field Measurements

At each sampling location, in-situ measurements of key physicochemical parameters were taken immediately after sample collection: Water Temperature (°C) and Electrolytic Conductivity (EC at 25°C) were measured using a handheld multiparameter probe. pH was determined on-site using a calibrated digital pH meter.

Sample Collection and Preservation

Water samples were collected in transparent, sterile polypropylene bottles (1 L capacity). Each bottle was filled completely to avoid headspace, minimizing the risk of oxidation or UV degradation of antibiotic residues. To prevent contamination, bottles were rinsed three times with the water from the respective sampling site before final collection. Samples were labeled clearly with the date, time, and site code. Immediately after collection, samples were placed in insulated coolers containing ice packs and transported to the laboratory for analysis. Some analyses were performed on the same day of sampling while others samples were stored at −20°C until further analysis to prevent degradation of antibiotic compounds.

Soxhlet Extraction Method

- 100ml of sample was extracted with 30ml of methanol for 3hrs.

- Crude extract obtained is evaporated using a rotary vacuum evaporator at 400c, just to dryness.

- Residue re-dissolved in an ammonium acetate buffer to a final volume of 2ml for cleanup.

Florisil Clean Up

- Packed column filled with 1 mL of ethanol, 1 mL of water and 1 mL of the ammonium acetate buffer, sequentially. Target compounds were eluted with 4 mL methanol and ammonium hydroxide solution 1 M (75:25, v/v). The eluates were evaporated to dryness at 37 ˝C, and the residue was re-dissolved in 300 µL of formic acid solution for chromatographic analysis.

Gas Chromatographic conditions for Antibiotics determination

The eluent were analyzed by Gas Chromatograph-Buck M910 scientific gas chromatography equipped with Flame ionization detector that allowed the detection of contaminants even at trace level concentrations (in the lower μg/kg range) from the matrix to which other detectors do not respond. The GC conditions used for the analysis were .capillary column coated with DB-5ms (30 m + 10 m EZ guard column x 0.25 mm internal diameter, 0.25 μm film thickness). The injector and detector temperature were set at 250 ºC and 280ºC respectively. The oven temperature was programmed as follows: 120 ºC held for 4 min, ramp at 10 ºC/ min to 180 ºC, held for 2 min, and finally ramp at 5 ºC/ min to 300 ºC. Nitrogen was used as carrier gas at a flow rate of 1.0 mL/ min and detector make-up gas of 29 Ml min-1. The injection volume of the GC was 10.0 μL. The total run time for a sample was 43 min.

The limit of quantification (LOQ) defined as the limit of detection divided by the sampling volume was of 0.0001 and 0.0003 %. The recovery efficiency of the method was evaluated by the analysis of filters spiked with known concentration of standard ANTIBIOTIC compounds. Most of the compounds provided high recoveries with mean values ranging between80 to 90%. Field and laboratory blanks were routinely analyzed for quality control. Blanks levels of individual analyses were normally very low and in most cases not detectable

Quantification of PAH residues.

The residue levels of antibiotics were quantitatively determined by the external standard method using peak area. Measurement was carried out within the linear range of the detector. The peak areas whose retention times coincided with the standards were extrapolated on their corresponding calibration curves to obtain the concentration

Preparation of standard

10ul and 20ul of Accu standard was injected in the chromatography and the retention time compared with retention time of standard.

RESULT

Figure 1: Google based map of Atavo River Akpugo generated in 2025

# Figure 2: Google based map of Atavo River Amodu generated in 2025

The concentrations of antibiotics detected in the head region of the river water samples are presented in Table 1. A total of seven antibiotics—penicillin, tetracycline, dicloxacillin, oxacillin, ampicillin, amoxicillin, and avilamycin were detected in this section of the river. Among the antibiotics detected, penicillin exhibited the highest concentration (0.18 µg/ml), indicating a relatively high level of contamination in the upstream portion of the river. Amoxicillin recorded the second-highest concentration at 0.14 µg/ml, followed closely by tetracycline with 0.13 µg/ml. Dicloxacillin and avilamycin were both present at 0.04 µg/ml, suggesting moderate contamination levels. In contrast, ampicillin and oxacillin were detected at relatively low concentrations of 0.01 µg/ml each.

Table 1: Concentration (µg/ml) of antibiotics in the examined head surface water sample (Akpugo)

| Antibiotics | Concentration |
| --- | --- |
| Penicillin | 0.18 |
| Tetracycline | 0.13 |
| Dicloxacillin | 0.04 |
| Oxacillin | 0.01 |
| Tylosin | - |
| Amoxicillin | 0.14 |
| Avilamycin | 0.04 |
| *Aldrin | - |
| Ampicillin | 0.01 |
| Macrolide | - |
| Gentamicin | - |
| *Quinoline | - |
| Colistin | - |
| Lincomycin | - |

The concentration of antibiotics detected in the mid-section of the river water samples is presented in [Table 2]. Among the antibiotics analyzed, tetracycline exhibited the highest concentration, measured at 0.10 µg/ml. This indicates that tetracycline is the most dominant antibiotic residue within this region of the river. The second highest concentration was recorded for lincomycin at 0.04 µg/ml, followed by colistin, which showed a concentration of 0.03 µg/ml. Both aldrin and gentamicin were detected at relatively low but equal concentrations of 0.01 µg/ml each. In total, five antibiotics were detected in the midstream samples. Notably, penicillin, dicloxacillin, oxacillin, ampicillin, amoxicillin, quinoline, avilamycin, tylosin, and macrolide were not detected in the mid-section of the river.

Table 2: Concentration (µg/ml) of antibiotics in the examined mid surface water sample (Akpugo)

| Antibiotics | Concentration |
| --- | --- |
| Penicillin | - |
| Tetracycline | 0.10 |
| Dicloxacillin | - |
| Oxacillin | - |
| Ampicillin | - |
| Amoxicillin | - |
| Avilamycin | - |
| *Aldrin | 0.01 |
| Tylosin | - |
| Macrolide | - |
| Gentamicin | 0.01 |
| *Quinoline | - |
| Colistin | 0.03 |
| Lincomycin | 0.04 |

*Aldrin is not an antibiotic but a pesticide it was detected during analysis of the sample. This means a total of 12 antibiotics were studied with 4 antibiotics being detected and 1 pesticide detected.

In the downstream region of the river, a total of seven antibiotics were detected as shown in [Table 3]. Among these, tetracycline recorded the highest concentration at 0.19 µg/ml, representing the most significant level of contamination observed. The concentrations of the detected antibiotics in descending order were as follows: tetracycline (0.19 µg/ml), penicillin (0.08 µg/ml), amoxicillin (0.05 µg/ml), dicloxacillin (0.04 µg/ml), ampicillin (0.03 µg/ml), avilamycin (0.03 µg/ml), and oxacillin (0.01 µg/ml). Other antibiotics, including tylosin, macrolide, gentamicin, colistin, and lincomycin, were not detected in the downstream samples.

Table 3: Concentration (µg/ml) of antibiotics in the examined downstream water sample (Akpugo)

| Antibiotics | Concentration |
| --- | --- |
| Penicillin | 0.08 |
| Tetracycline | 0.19 |
| Dicloxacillin | 0.04 |
| Oxacillin | 0.01 |
| Ampicillin | 0.03 |
| Amoxicillin | 0.05 |
| Avilamycin | 0.03 |
| *Aldrin | - |
| Tylosin | - |
| Macrolide | - |
| *Quinoline | - |
| Gentamicin | - |
| Colistin | - |
| Lincomycin | - |

The spring water sample contained little to no detectable levels of antibiotics. Among all the antibiotics analyzed, only lincomycin showed a measurable concentration of 0.01 mg/ml, as presented in [Table 4]. Other antibiotics, including tetracycline, macrolide, avilamycin, colistin, tylosin, penicillin, and ampicillin, were not detected. Additionally, an impurity identified as quinoline was detected in the sample at a concentration of 0.01 mg/ml, identical to that of lincomycin.

Table 4: Concentration (µg/ml) of antibiotics in the examined spring water sample (Akpugo)

| Antibiotics | Concentration |
| --- | --- |
| Penicillin | 0.00 |
| Tetracycline | 0.00 |
| Dicloxacillin | - |
| Oxacillin | - |
| Ampicillin | 0.00 |
| Amoxicillin | - |
| Avilamycin | 0.00 |
| *Aldrin | - |
| Tylosin | 0.00 |
| Macrolide | 0.00 |
| *Quinoline | 0.01 |
| Gentamicin | - |
| Colistin | 0.00 |
| Lincomycin | 0.01 |

*Quinoline is not an antibiotic but a major compound present in antimalarial drugs; it was detected in the sample during analysis.

In the head region of the river (upstream), a total of six different antibiotic residues were detected in the analyzed water sample, as presented in [Table 5]. Among these, tetracycline exhibited the highest concentration at 0.50 µg/ml, representing the most dominant antibiotic detected in this region. Several other antibiotics selected for this study were not detected in the sample. The antibiotics identified in the head region, arranged in descending order of their concentrations, include: tetracycline (0.50 µg/ml), colistin (0.10 µg/ml), avilamycin (0.04 µg/ml), tylosin (0.04 µg/ml), and ampicillin (0.01 µg/ml).

Table 5: Concentration (µg/ml) of antibiotics in the examined Upstream (Amodu)

| Antibiotics | Concentration |
| --- | --- |
| Penicillin | - |
| Tetracycline | 0.50 |
| Dicloxacillin | - |
| Oxacillin | - |
| Ampicillin | 0.01 |
| Amoxicillin | - |
| Avilamycin | 0.04 |
| Tylosin | 0.04 |
| *Aldrin | - |
| Gentamicin | - |
| Colistin | 0.10 |
| Macrolide | - |
| *Quinoline | 0.03 |
| Lycomycin | - |

The concentration of antibiotics in the midstream region of the river is presented in Table 6. A total of five antibiotics were detected in this section, representing the lowest number of antibiotics found among all the sampled regions. Penicillin recorded the highest concentration at 0.13 µg/ml, followed by lincomycin (0.10 µg/ml), colistin (0.03 µg/ml), amoxicillin (0.02 µg/ml), and gentamicin (0.01 µg/ml). Antibiotics such as tetracycline, dicloxacillin, oxacillin, ampicillin, avilamycin, tylosin, and macrolide were not detected in this region.

Table 6: Concentration (µg/ml) of antibiotics in the examined mid region (Amodu)

| Antibiotics | Concentration |
| --- | --- |
| Penicillin | 0.13 |
| Tetracycline | - |
| Dicloxacillin | - |
| Oxacillin | - |
| Ampicillin | - |
| Amoxicillin | 0.02 |
| Avilamycin | - |
| *Aldrin | - |
| Tylosin | - |
| Macrolide | - |
| *Quinoline | - |
| Gentamicin | 0.01 |
| Colistin | 0.03 |
| Lincomycin | 0.10 |

The final sample collected from the tail region (downstream) of the river contained six different antibiotics, as presented in Table 7. Interestingly, the antibiotics detected in this section were identical to those found in the head region, though their concentrations varied. Tetracycline recorded the highest concentration at 0.65 µg/ml, followed by colistin at 0.10 µg/ml. Tylosin, avilamycin, and quinoline were detected at concentrations of 0.06 µg/ml, 0.04 µg/ml, and 0.03 µg/ml, respectively, while ampicillin had the lowest concentration of 0.02 µg/ml.

Table 7: Concentration (µg/ml) of antibiotics in the examined tail region (Amodu)

| Antibiotics | Concentration |
| --- | --- |
| Penicillin | - |
| Tetracycline | 0.65 |
| Dicloxacillin | - |
| Oxacillin | - |
| Ampicillin | 0.02 |
| Amoxicillin | - |
| Avilamycin | 0.04 |
| *Aldrin | - |
| Tylosin | 0.06 |
| Macrolide | - |
| *Quinoline | 0.03 |
| Gentamicin | - |
| Colistin | 0.10 |
| Lincomycin | - |

DISCUSSION

Antibiotics are natural, semi-synthetic, or synthetic compounds capable of combating bacterial infections by either killing the bacteria (bactericidal) or inhibiting their growth and reproduction (bacteriostatic), thereby aiding the body’s natural defense mechanisms in eliminating them (Bansal, 2022). They have been widely used across the world in the prevention and treatment of diseases in both humans and animals (Baralla et al., 2021). However, the presence of antibiotics in environmental or spring water poses a significant threat to human health. Water is considered a vital natural resource essential for various human activities, including drinking, irrigation of food crops, and recreational purposes, among others (Felis et al., 2020, Emamgholizadeh, 2014).

In this study, tetracycline was detected in all river water samples collected from both Akpugo and Amodu, with a net concentrations of 0.14 μg/ml and 0.60 μg/ml, respectively. It recorded the highest concentration among all antibiotics analyzed in samples from both communities but was not detected in the drinking water sample. This finding is consistent with the report by Lenart-Borón et al. (2022), who identified tetracycline as one of the most persistent antibiotics in the aquatic environment of the Białka River. Tetracycline is known for its ability to inhibit protein synthesis in bacteria and treat a wide range of bacterial infections (Ahmad et al., 2021). However, its presence in water bodies poses significant environmental and public health concerns. It can accumulate through the food chain, induce toxicity in microbial communities, promote the development and dissemination of antibiotic resistance, threaten the safety of drinking and irrigation water, and disrupt the natural microbial flora in the human intestine (Amangelsin et al., 2023).

Amoxicillin was detected in the surface water samples, specifically in the downstream (0.05 µg/mL) and head (0.14 µg/mL) regions of the river at Akpugo. However, it was completely absent in all the samples collected from the river at Amodu. This observation aligns with the findings of Akhter et al. (2023), who reported an amoxicillin concentration of 1.2347 ng/mL in surface water samples. Amoxicillin was also not detected in the spring water. According to Armando et al. (2016), the presence of such compounds in water bodies can have toxic effects on aquatic organisms across different trophic levels, potentially leading to ecological imbalances. These effects may include the bioaccumulation of antibiotics in fish muscle tissues, which could result in their entry into the food chain and the inadvertent consumption of these drugs by humans, possibly causing adverse health effects such as immunoallergic responses.

Ampicillin was detected in the surface water samples collected from the river at Akpugo, specifically in the head and downstream regions, with concentrations of 0.01 µg/ml and 0.03 µg/ml, respectively. Similarly, in samples obtained from the river at Amodu, ampicillin was present at relatively low concentrations—0.01 µg/ml in the head region and 0.02 µg/ml in the tail region. No traces of ampicillin were detected in the spring water sample. This observation aligns with the findings of Squiggins et al. (2024), who reported that the presence of ampicillin-resistant coliforms in urban floodwaters increases the potential risk of human exposure to these pathogenic microorganisms.

The concentration of gentamicin in the mid-surface water samples from Akpugo was found to be as low as 0.01 µg/ml. This finding contrasts with the study by Lenart-Borón et al. 2022, where gentamicin was among the few antibiotics not detected. In the present study, gentamicin was also detected in the mid-region of the river at Amodu, with a similarly low concentration of 0.01 µg/ml. Antibiotics remain a major cause of concern due to their persistent nature, incomplete metabolism, and ability to move easily through various environmental compartments (Kümmerer, 2009).

Penicillins are organic acids characterized by a β-lactam ring attached to a side chain and a thiazolidine ring. They act by inhibiting bacterial cell wall synthesis, specifically by blocking the transpeptidase reaction essential for cell wall formation (Lobanovska and Pilla, 2017). In this study, penicillin recorded the second-highest overall concentration among all antibiotics detected in samples from Akpugo, with a value of 0.13 µg/ml. The antibiotic was also identified in the mid-region water sample from Amodu at the same concentration of 0.13 µg/ml, which represents a relatively high level compared to other antibiotics detected in this research. The persistence of antibiotics in aquatic environments is often due to their rate of introduction exceeding their rate of degradation or removal. Prolonged exposure to sub-inhibitory concentrations of antibiotics in water systems can induce genomic changes in microorganisms, facilitating the development of antibiotic resistance and the transfer of resistance genes (Kulik et al., 2023). The primary sources of these antibiotics in the environment include domestic misuse, agricultural and aquaculture practices, as well as clinical waste (Sosa-Hernández et al., 2021).

Approximately 50–90% of antibiotics consumed by humans and animals undergo only partial metabolism. Consequently, a significant portion of these compounds is excreted in both their parent and metabolite forms through feces and urine, leading to the widespread presence of antibiotics in wastewater and surface water bodies (Tran et al., 2019). Dicloxacillin and Oxacillin were detected in the samples gotten from the river at akpugo with a net concentration of of 0.04 µg/ml  and .01 μg/ml, respectively. Oxacillin at high levels can cause liver injury by elevations in serum ALT (Chalasani et al., 2020). Oxacillin and Dicloxacillin was completely absent in all samples collected from the river at Amodu, as well as in the spring water samples.

Traces of Avilamycin were detected in the surface water samples, specifically in the head and downstream regions of the river at Akpugo. Mean while, the river at Amodu showed traces of Avilamycin and tylosin in both the head and tail regions, No traces of the antibiotic were detected in the spring water samples. According to Koch et al. (2021), the excessive use of antibiotics can negatively impact the gene pool and food chain, leading to the emergence of antibiotic-resistant and multi-drug-resistant bacteria.

Colistin was detected only in the mid-region of the surface water sample from the river at Akpugo, with a concentration of 0.03 µg/ml. No traces of the antibiotic were found in the drinking water samples. In contrast, Colistin was one of the two antibiotics detected in all samples collected from the river at Amodu. It was present in the head, mid, and tail regions, with relatively high concentrations of 0.10 µg/ml recorded at both the head and tail regions, while the mid-region had a lower concentration of 0.03 µg/ml. Previous research has shown that the primary environmental concern associated with Colistin contamination in water bodies is its ability to promote the spread of antibiotic resistance genes, such as mcr genes, among bacteria—posing a significant public health risk (Osisiogu et al., 2025).

Lincomycin is an antibiotic commonly used in veterinary medicine (Mehrtens et al., 2021). In this study, Lincomycin was detected in the spring water consumed by the community at a concentration of 0.01 µg/ml, representing the highest antibiotic concentration among all spring water samples. Owing to the scarcity of potable water, particularly in rural areas, people often rely on any available water source that appears clean for drinking, washing, and recreational activities (Emurotu and Habib, 2019). In the river at Amodu, Lincomycin was detected only in the mid-region, where it recorded a concentration of 0.10 µg/ml. The occurrence of Lincomycin in environmental water has been shown to promote antimicrobial resistance, thereby reducing the effectiveness of antibiotics in treating infections and potentially disrupting microbial communities and ecological processes (Mehrtens et al., 2021).

Aldrin and Quinoline are not antibiotics but are classified as an organochlorine pesticide and a compound found in antimalarial drugs, respectively. In this study, Aldrin was detected in the mid-region of the surface water sample from the river at Akpugo, with a concentration of 0.01 µg/ml. Quinoline, on the other hand, was detected in the spring water at Akpugo at a concentration of 0.01 µg/ml. Traces of Quinoline were also found in the head and tail water samples collected from the river at Amodu. According to Luo et al. (2020), Quinoline exhibits carcinogenic, teratogenic, and mutagenic effects in humans through bioaccumulation in the food chain.

# Conclusion

This study investigated the presence and concentration of antibiotics in both the Atavu River and the spring water located at Akpugo and Amodu. Of the 12 antibiotics analyzed, 11 were detected. Among these, tetracycline was the most frequently found and exhibited the highest concentration, followed by penicillin, which is widely utilized in veterinary medicine and various other applications. The distribution of these antibiotics throughout the river likely contributed to their presence in the spring water. The detection of antibiotics in the spring water, which serves as a primary source of drinking water, is particularly alarming, as prolonged exposure may lead to the development of antibiotic resistance and other health conditions that pose serious threats to human life. However, further studies are required to carry out this experiment in different seasons of the year.

Disclaimer (Artificial intelligence)

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

References:

- Acharya, L.K., Paramaguru, P.K., Tripathi, K. (2025). Pesticide contamination in groundwater: processes, risks, and mitigation strategies. Discovery Agriculture, 3: 152.

- Adefegha, A. (2019). Antibiotics and drug pharmacology. Acta Sci Pharm Sci, 3, 43-9.

- Ahmad, F., Zhu, D. & Sun, J. (2021). Environmental fate of tetracycline antibiotics: degradation pathway mechanisms, challenges, and perspectives. Environmental Sciences Europe, 33:64.

- Akhter, S.; Bhat, M.A.; Ahmed, S.; Siddiqi, W.A.; Ahmad, S.; Shrimal, H. (2023). Profiling of Antibiotic Residues in Surface Water of River Yamuna Stretch Passing through Delhi, India. Water. 15, 527.

- Akhtar, N., Syakir Ishak, M. I., Bhawani, S. A., & Umar, K. (2021). Various Natural and Anthropogenic Factors Responsible for Water Quality Degradation: A Review. Water,13(19), 2660. https://doi.org/10.3390/w13192660

- AL-Harmoosh, R. A., Jarallah, E. M., AL-Shamari, A.M and AL-khafaji, H. M. (2017). Detection of Efflux Pumps Genes in Clinical Isolates of Acinetobacter baumannii. Research J. Pharm. And Tech. 10(12):4231-4236.

- Aljanaby A.A., Witwit I.N., Mubark H.M., Al-Labban H.M.Y. (2019). Synthesis and characterization of new imidazole azo ligand with some of transition metal ions, and their biological effect on two pathogenic bacteria of burn patients. International Journal of Research in Pharmaceutical Sciences, 10(3):1847-1856.

- Aljofan, M., Semenova, Y., Amangelsin, Y., Dadar, M., Bjorklund, G. (2023). The Impact of Tetracycline pollution on the Aquatic Environment and Removal Strategies, 12(3): 440.

- Amangelsin, Y., Semenova, Y., Dadar, M., Aljofan, M and Geir, B. (2023). The Impact of Tetracycline Pollution on the Aquatic Environment and Removal Strategies. Antibiotics (Basel).12(3):440.

- Andersson, D. I., Balaban, N. Q., Baquero, F., Courvalin, P., Glaser, P., Gophna, U., and Tønjum, T. (2020). Antibiotic resistance: turning evolutionary principles into clinical reality. FEMS Microbiology Reviews.

- Armando, E., Leobardo, M. G., Marcela, G., Hariz, I., Octavio, D. and Nely, S. (2016). Amoxicillin in the Aquatic Environment, Its Fate and Environmental Risk. Hazardous Factors to Living Species, DOI: 10.5772/62049.

- Baralla, E.; Demontis, M.P.; Dessì, F.; Varoni, M.V. (2021). An Overview of Antibiotics as Emerging Contaminants: Occurrence in Bivalves as Biomonitoring Organisms. Anim. Open Access J. 11, 3239.

- Basnal, O.P. (2022). International Journal of Frontiers in Biology and Pharmacy Research, 2(1): 057-083.

- Chalasani N, Fontana RJ, Bonkovsky HL, Watkins PB, Davern T, Serrano J, Yang H, Rochon J. (2020). Drug Induced Liver Injury Network (DILIN). Causes, clinical features, and outcomes from a prospective study of drug-induced liver injury in the United States. Gastroenterology, 135:1924–34.

- Chukwudi, C. U. (2016). rRNA binding sites and the molecular mechanism of action of the tetracyclines. Antimicrobial agents and chemotherapy, 60(8), 4433-4441.

- Dabhi, M., Patel, R., Shah, V., Soni, R., Saraf, M., Rawal, R., & Goswami, D. (2024). Penicillin-binding proteins: the master builders and breakers of bacterial cell walls and its interaction with β-lactam antibiotics. Journal of Proteins and Proteomics, 15(2), 215-232.

- Emamgholizadeh S; Kashi, HI;Zalaghi, M.E. (2014). Prediction of water quality parameters of Karoon River (Iran) by artificial intelligence-based models”, Int. J. Environ. Sci. Technol., 11(1): 645-656

- Emurotu, J.E and Habib, L.O. (2019). Journal of Applied Science and Environmental Management, 23(11): 1995-2002.

- Etebu, E., and Arikekpar, I. (2016). Antibiotics: Classification and mechanisms of action with emphasis on molecular perspectives. Int. J. Appl. Microbiol. Biotechnol. Res, 4: 90-101.

- Felis, E.; Kalka, J.; Sochacki, A.; Kowalska, K.; Bajkacz, S.; Harnisz, M.; Korzeniewska, E. (2020). Antimicrobial Pharmaceuticals in the Aquatic Environment—Occurrence and Environmental Implications. Eur. J. Pharmacol. 866, 172813.

- Hasan, H.T. and Al-Harmoosh, A.R. (2020). Mechanisms of Antibiotics Resistance in Bacteria. A multifaceted review journal in the field of pharmacy. 11(6): 817-823.

- Jahne, M. A., Rogers, S. W., Ramler, I. P., Holder, E., and Hayes, G. (2015). Hierarchal clustering yields insight into multidrug-resistant bacteria isolated from a cattle feedlot wastewater treatment system. Environmental monitoring and assessment, 187(1):4168.

- Jimenez, C.E.P., Keestra, S., Tandon, P., Cumming, O., Pickering, A. J., Moodley, A., Chandler, I. R. (2023). Biosecurity and water, sanitation, and hygiene (WASH) interventions in animal agricultural settings for reducing infection burden, antibiotic use, and antibiotic resistance: a One Health systematic review. The Lancet Planetary Health, 7(5): 418-434.

- Koch, N., Islam, N. F., Sonowal, S., Prasad, R. and Sarma, H. (2021). Environmental antibiotics and resistance genes as emerging contaminants: Methods of detection and bioremediation. Current Research in Microbial Sciences, 2: 100027.

- Kulik, K., Lenart-Boroń, A., & Wyrzykowska, K. (2023). Impact of Antibiotic Pollution on the Bacterial Population within Surface Water with Special Focus on Mountain Rivers. Water, 15(5), 975. https://doi.org/10.3390/w15050975.

- Kümmerer, K. (2009). Antibiotics in the aquatic environment—A review—Part I. Chemosphere, 75, 417–434.

- Lenart-Boro´n, A.; Prajsnar, J.; Guzik, M.; Boro´n, P.; Grad, B.; Zelazny, M. (2022). Antibiotics in Groundwater and River Water of Białka—A Pristine Mountain River. Appl. Sci. 12, 12743.

- Lobanovska, M. and Pilla, G. (2017). Penicillin’s Discovery and Antibiotic Resistance. Yale Journal of Biology and Medicine, 90(1):135–145.

- Luo, Y., Yue, X., Wei, P., Zhou, A., Kong, X. and Alimzhanova, S. (2020). A state-of-the-art review of quinoline degradation and technical bottlenecks. Science of The Total Environment, 747: 141136.

- Majeed, H. T., and Aljanaby, A. A. J. (2019). Antibiotic Susceptibility Patterns and Prevalence of Some Extended Spectrum Beta-Lactamases Genes in GramNegative Bacteria Isolated from Patients Infected with Urinary Tract Infections in Al-Najaf City, Iraq. Avicenna Journal of Medical Biotechnology, 11(2):192.

- Mehrtens, A., Licha, T. and Burke, V. (2021). Occurrence, effects and behaviour of the antibiotic lincomycin in the agricultural and aquatic environment – A review. Science of The Total Environment, 778: 146306.

- Osisiogu, E. U., Mahmoud, F. C., Waqas, F. B., Singh, B., Feglo, P. K. and Duedu, K. O. (2025).  Environmental mediation of colistin resistance in the African context. A systematic scoping review. Journal of Global Antimicrobial Resistance, 41: 39-43.

- Sosa-Hernández, J.E.; Rodas-Zuluaga, L.I.; López-Pacheco, I.Y.; Melchor-Martínez, E.M.; Aghalari, Z.; Limón, D.S.; Iqbal, H.M.N.; Parra-Saldívar, R. (2021). Sources of Antibiotics Pollutants in the Aquatic Environment under SARS-CoV-2 Pandemic Situation. Case Stud. Chem. Environ. Eng. 4, 100127.

- Squiggins KT, Fullerton H and Vulava VM (2024). The presence of ampicillin-resistant coliforms in urban floodwaters of a coastal city in the southeastern United States. Frontier Water, 6:1359196. doi: 10.3389/frwa.2024.1359196.

- Tran, N.H.; Hoang, L.; Nghiem, L.D.; Nguyen, N.M.H.; Ngo, H.H.; Guo, W.; Trinh, Q.T.; Mai, N.H.; Chen, H.; Nguyen, D.D. (2019). Science of the Total Environment Occurrence and risk assessment of multiple classes of antibiotics in urban canals and lakes in Hanoi, Vietnam. Sci. Total Environ. 692, 157–174.

- Waglechner, N., and Wright, G. D. (2017). Antibiotic. BMC biology, 15(1): 1-8