Utilization of Jackfruit Waste for the Development of Sustainable Animal Feed

ABSTRACT

Jackfruit (Artocarpus heterophyllus) is one of the largest tropical fruits and generates a substantial quantity of waste, accounting for nearly 65–75% of its total weight in the form of non-edible components such as peel, core and perianth. The improper disposal of this agro-waste poses environmental challenges, while its rich nutritional potential remains under-utilized. The present study was undertaken to develop a sustainable and value-added process for converting jackfruit waste into a nutritive feed powder. Jackfruit waste obtained from the J33 cultivar was thoroughly cleaned, uniformly cut, and blended with maize flour to enhance its nutritional and functional properties. The prepared mixture was subjected to dehydration using two different drying methods, namely a solar-cabinet dryer and traditional open-sun drying, to evaluate their effectiveness in moisture removal and quality retention. The resulting feed powder was analysed for its physical, functional, and proximate properties to assess its suitability as a value-added feed ingredient. The findings of this study highlight the potential of jackfruit waste as a low-cost, eco-friendly, and nutritionally beneficial resource for feed formulation, thereby contributing to waste reduction and sustainable agro-processing practices.

Keywords: animal feed; Jackfruit waste powder; proximate composition; physical properties; solar drying.

1. INTRODUCTION

Jackfruit (Artocarpus heterophyllus) is a major tropical fruit crop in India, producing over 18 lakh tonnes annually, and plays a crucial role in nutrition, economy, and rural employment (Mashwani et al., 2020; Swami et al., 2012). The edible portion represents only 25–35 % of the fruit; the remaining 65–75 % comprises peel, core, and perianth, which are generally discarded as waste (Li et al., 2019). These non-edible fractions are rich in carbohydrates, fiber, minerals, and minor proteins, making them a promising resource for livestock feed (Munishamanna et al., 2019; Ranasinghe et al., 2019).

Increasing costs of conventional feed ingredients such as maize bran, combined with environmental concerns related to organic waste disposal, have necessitated the exploration of alternative feed sources (MAAF, 2008; FAO, 2020). Converting jackfruit residues into feed powder offers multiple advantages: it provides a stable, nutrient-rich supplement, enhances feed palatability, reduces environmental impact, and supports sustainable livestock production (Prasad et al., 2018).

Proper processing, including chopping, drying, and grinding, is essential to maintain nutrient content, improve flow properties, and ensure long-term storage stability (Kamble and Dombale, 2015; Ajey, 2013). While previous studies have explored the nutritional potential of jackfruit residues and their use in ruminant diets, systematic evaluation of powder preparation, physical characteristics, and proximate composition is limited. This study aims to develop jackfruit waste powder using solar-cabinet and open-sun drying methods, evaluate its physical and functional properties, and establish its potential as a low-cost, sustainable feed for livestock.

MATERIALS AND METHODS

Raw material collection

The jackfruit J33 (Tekam yellow) were procured from Ratnagiri Village, Sri Sathya Sai District, Andhra Pradesh.

Preparation of Jackfruit Waste Powder

The jackfruit outer peel, core and rag were cut by knives and bulbs are removed manually. By using hammer mill, jackfruit waste & maize are ground into fine powder. Using a dough kneader, prepare the dough by mixing of maize powder and jackfruit waste. The fine mix was spread into the aluminium trays, then the trays were kept in the solar cabinet dryer and open sun drying. Process flow chart for the preparation of jackfruit waste powder is represented in Figure 1. After 6 to 7 h of drying using a solar cabinet dryer and open sun drying, takes out trays and powder were collected.

JackFruit

Peeling and Deseeding

Cutting and slicing

Grinding (Hammer mill)

(Adding of water)

Mixing

(Addition of jackfruit waste with maize powder)

Spread the jackfruit waste directly on trays

Drying (Solar Cabinet dryer & Open Sun drying)

Dried jackfruit waste powder

Fig. 1 Process flow chart for the Preparation of Jackfruit Waste Powder

2.3 Physical Properties

2.3.1 Bulk Density

Bulk density is measured by adding 2g of powder into empty dry cylindrical vessel and the bulk density is determined by based on the following formula (Jyothi et al., 2022).

ρb (g/cc) =Mass of the sample Volume of the sample        	                                            …(1)

Where,

ρb = Bulk density

2.3.2 Tapped Density

Tapped density is the ratio of the mass of the powder to the volume occupied by the powder after it has been tapped for a definite period of time (Jyothi et al., 2022).

Ρt (g/cc) = Mass of the sampleMinimum volume occupied after tapping                                              … (2)

Where,

ρt = Tapped density

2.3.3 Porosity

The porosity of the sample was estimated using the relationship between the bulk and actual density of the powder (Smita et al., 2019). The porosity of samples was determined according to Equation (3).

Porosity (%) = 1-ρbρt ×100                                                      … (3)

2.3.4 Angle of Repose

The angle of repose of powder is measured by the fixed cone height method. The funnel with an internal stem diameter of 5mm is placed over a glass slide. Particles are allowed to flow gently through the funnel until a cone is formed, then up to the cone height remains constant (Mosgoeller et al., 2012). The angle of the horizontal cone is recorded; this test should be performed in triplicate for each sample.

Θ =tan-1(2hd) degree (º)                                                              … (4)

Where,

h = height

d = diameter

2.4 Functional Properties

2.4.1 Flowability

Flowability was determined as the ratio of tapped density to the bulk density  
(Pugliese et al., 2017).

Flowability  =  ρtρb                                                                       …(5)

Where,

ρt = Tapped density

ρb = Bulk density

2.4.2 Carr Index

The Carr index (Carr’s index or Carr’s compressibility Index) is an indication of the compressibility of a powder. It is determined by the ratio of the difference between tapped and bulk density to tapped density (Patil et al., 2014).

CI (%) = 100×ρt-ρbρt          				… (6)

2.4.3 Water absorption capacity

Water absorption capacity was determined (Mawardii et al., 2017). An accurately weighed two grams of dried jackfruit waste powder and pellets were weighed (initial weight) into 15 ml beakers and 13 ml of distilled water was added at room temperature and kept for 
2 hours and the water was filtered through filter paper until all the water was drained out and the sticky water was removed by laboratory tissue paper and finally, the weight of water absorbed by the samples was taken (final weight). The water absorption capacities were determined according to the formula.

Water absorption capacity = Final weight of the sample-Initial weight of the sample Initial weight of the sample … (7)

2.5 Proximate Composition

The jackfruit waste powder was analysed for proximate analysis such as moisture, protein, total fat, total ash, carbohydrate and energy, expressed in percentage. The analysis was done at Pristine Laboratories, Bangalore (AOAC, 2016).

3. RESULTS AND DISCUSSIONS

The present study investigated the physical, functional and nutritional characteristics of Artocarpus heterophyllus waste powder prepared using a combination of jackfruit peel, core and rag along with maize flour. The results were critically analysed and compared with previous studies to evaluate their suitability as a sustainable livestock feed ingredient.

3.1 Physical Properties

The physical properties of jackfruit waste significantly influence its handling, storage, and incorporation into feed formulations. The bulk density (0.15 g/cm3) obtained in this study is considerably lower than the values reported for jackfruit peel powder in earlier studies (Jyothi et al., 2022). This lower bulk density indicates that the developed powder is highly porous and occupies a larger volume per unit mass. Such a characteristic is advantageous in ruminant nutrition as it enhances rumen fill and promotes satiety, which can improve feed intake behaviour.

In contrast, the tapped density (0.85 g/cm3) observed was relatively high, indicating that the powder undergoes significant compaction when subjected to mechanical vibration. The large difference between bulk and tapped density reflects the compressibility of the material, which is a common feature of fibrous agro-residues. Similar trends have been reported in studies on fruit waste powders, where high compressibility is associated with irregular particle structure and fibrous composition (Patil et al., 2014).

The porosity (81.49%) reported in this study is significantly higher than the previously documented values for jackfruit peel powder (10-15%) (Smita et al., 2019). This high porosity indicates the presence of substantial inter-particle void spaces, which enhance water retention and facilitate oxygen diffusion. From a nutritional perspective, high porosity is beneficial as it improves microbial accessibility during rumen fermentation, thereby enhancing digestibility. The high porosity can be attributed to the inclusion of multiple jackfruit components (peel, core, and rag), which contain lignocellulosic fibres that contribute to a loose and open structure. However, excessive porosity may also result in poor flowability and segregation during storage, which can affect feed uniformity.

The angle of repose (42.38°) further confirms the poor characteristics of the powder. According to standard classification, powders with an angle of repose greater than 40° exhibit cohesive behaviour and poor flowability. This value was higher than those reported in previous studies (28-32°) of jackfruit peel powder (Jyothi et al., 2022), indicating that the present formulation is more cohesive. The higher angle may be due to the presence of fibrous particles, irregular shapes, and moisture-binding components such as pectin. Table 1 shows the physical parameters of the jackfruit waste (outer peel, core and rag).

TABLE 1 | Physical properties of A. heterophyllus waste powder

| Properties | Values |
| --- | --- |
| Bulk density (g/cm3) | 0.15 |
| Tapped density (g/cm3) | 0.85 |
| Porosity (%) | 81.49 |
| Angle of repose (º) | 42.38º |

3.2 Functional Properties

Functional properties provide further insight into the handling and performance of the powder. The flowability value (5.40) indicates extremely poor flow behaviour, which consistent with the high angle of repose observed. The Carr index (81.44%) is exceptionally high compared to the acceptable range (<15% for excellent flow and <25% for good flow), indicating very high compressibility and cohesiveness. Similar findings have been reported for fibrous food powders, where Carr index values are attributed to inter-particle friction and electrostatic interaction (Pugliese et al., 2017).

The water absorption capacity (0.6 g/g) is a significant positive attribute. This moderate value indicates that the powder can absorb and retain water effectively, which is beneficial for animal feed applications. Water absorption enhances feed palatability and facilitates microbial activity in the rumen, leading to improved digestion and nutrient utilisation. Previous studies on jackfruit by-products have reported similar hydration properties due to the presence of dietary fibre and polysaccharides (Rahman et al., 2021). Compared to highly starchy materials, the moderate water absorption observed in this study prevents excessive swelling and stickiness, ensuring better handling and feed consistency. Table 2 shows the functional properties of jackfruit waste powder.

TABLE 2 | Functional properties of A. heterophyllus waste powder

| Properties | Values |
| --- | --- |
| Flowability | 5.40 |
| Carr index (%) | 81.44 |
| Water absorption capacity (g/g) | 0.64 |

3.3 Proximate composition

The proximate composition of the jackfruit waste powder demonstrates its nutritional suitability as a feed ingredient. The moisture content (5.72%) is within the safe storage range and comparable to previously reported values (7.63%) for jackfruit seed flour (Akter et al., 2019). Low moisture content is critical for preventing microbial growth and ensuring long shelf life, making the powder suitable for storage and transportation under ambient conditions.

The protein content (9.35 g/100g) indicates that the jackfruit waste can serve as a moderate protein source in animal feed. This value is consistent with earlier reports for jackfruit seed flour (10.26%) (Akter et al., 2019), although it is lower than values achieved through fermentation (20-23%). This suggests that further processing techniques, such as microbial fermentation or supplementation with nitrogen sources, could enhance the protein content and improve the nutritional quality of the feed.

The fat content (4.25 g/100g) contributes to the overall energy density of the feed. This value is comparable to earlier findings (4-6%) and indicates that the powder provides a moderate amount of lipids without posing risks of excessive fat intake. Lipids are essential for energy supply and absorption of fat-soluble vitamins, making this component nutritionally important.

The ash content (2.98 g/100g) reflects the mineral content of the powder. Similar reported values for jackfruit seed flour (2.35%) (Akter et al., 2019), it still indicates the presence of essential minerals required for physiological functions such as bone development, enzyme activity, and electrolyte balance. Variation in ash content may be due to the differences in raw material composition, soil condition, and processing methods.

A major highlight of the study is the high carbohydrate content (77.70 g/100g), which aligns well with previous studies reporting values between 78.65% for jackfruit seed flour (Akter et al., 2019). These high carbohydrates are mainly present in the form of starch and structural polysaccharides, which are readily fermented by rumen microorganisms to produce volatile fatty acids, the primary energy sources for ruminants.

The calculated energy value (386 kcal/100g) further confirms the suitability of the powder as an energy-dense ingredient. This value is comparable to the agro-industrial by-products used in animal feeding, such as rice bran and maize by-products. This inclusion of maize flour in the formulation may have contributed to the enhanced energy content, making the feed, more nutritionally balanced. Table 3 shows the proximate composition of jackfruit waste powder.

TABLE 3 | Proximate composition of A. heterophyllus waste powder

| ## Parameters | ## Results |
| --- | --- |
| ## Moisture Content (g/100g) | ## 5.72 |
| ## Protein (g/100g) | ## 9.35 |
| ## Total fat (g/100g) | ## 4.25 |
| ## Total ash (g/100g) | ## 2.98 |
| ## Carbohydrates (g/100g) | ## 77.70 |
| ## Energy (K Cal/100g) | ## 386.00 |

3.4 Evaluation of feed on field level

An important practical aspect of this study is the field-level evaluation, where the jackfruit waste powder was fed to a cow along with green fodder. The observed increase in milk yield from 3 L to 4 L per day suggests a positive impact of the feed on animal productivity. This improvement may be attributed to the additional energy and nutrients provided by the jackfruit waste powder. Similar findings have been reported in studies where unconventional feed resources improved milk yield and overall animal performance (Singh et al., 2022). However, it should be noted that this observation is preliminary and further controlled studies with larger sample sizes are required to validate the results.

Plate 1: Feeding jackfruit waste powder to cow

4. CONCLUSION

The present study demonstrates that jackfruit waste can be effectively converted into value-added feed powder with significant potential for livestock nutrition. Solar cabinet drying is recommended for efficient drying and improved quality. The developed product exhibited high carbohydrate content, moderate protein and low moisture, indicating good energy value, nutritional adequacy, and storage stability. The moderate water absorption capacity further supports its suitability for improving feed palatability and digestibility in ruminants.

However, the powder showed poor flow properties, characterised by low bulk density, high porosity, and elevated Carr index, which may limit its direct handling and processing. These challenges can be addressed through suitable processing techniques, such as pelletizing or blending.

Preliminary field evaluation indicated an increase in milk yield from 3 L to 4 L/day, suggesting its potential positive impact on animal productivity. Overall, jackfruit waste powder represents a cost-effective, eco-friendly feed alternative, contributing to sustainable livestock production and agro-waste management. Further studies on large-scale feeding trials and processing optimisation are recommended.

Disclaimer (Artificial intelligence)

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

REFERENCES

Adeyemi, O.A. and Sani, I., 2021. Agro-waste utilization in animal nutrition. Sustainability, 13(5), 2541.

Ajey, G., 2013, Microbial processing of jackfruit waste as animal feed (Doctoral dissertation, University of Agricultural Sciences, GKVK).

Akter, F. and Haque, M.A., 2019. Jackfruit waste: a promising source of food and feed. Annals of Bangladesh Agriculture, 23(1), 91-102.

AOAC., 2016, Official Methods of Analysis, 18th edn. Association of Official Analytical Chemists, Washington DC.

FAO., 2020, Agro-industrial residues: Utilization and sustainability in livestock feeding. FAO Technical Paper. Rome, Italy.

Hassan, M. et al., 2023. Valorization of food waste into animal feed. Waste Management, 156, 1–12.

Jyothi, G. S., Manjunath, R., Suresha, K.  B. and Ambika, D. S., 2022, Production of spray dried jackfruit powder from enzymatic liquefied juice. The Pharma Journal, 11(10): 1626-1633.

Kamble, A. K. and Dombale, R. L., 2015, Preparation of ready to use powder from tomato using solar cabinet dryer integrated with heat storage system. Journal of Ready to Eat Food, 2(1): 06-17.

Kumar, V. et al., 2021. Nutritional evaluation of fruit waste for ruminant feeding. Bioresource Technology Reports, 15, 100771.

Li, W. J., Fan, Z. G., Wu, Y. Y., Jiang, Z. G. and Shi, R. C., 2019, Eco‐friendly extraction and physicochemical properties of pectin from jackfruit peel waste with subcritical water. Journal of the Science of Food and Agriculture, 99(12): 5283-5292.Mashwani, Z. U., 2020,Environment,climate change and biodiversity. In: Environment,	Climate, Plantand Vegetation Growth. Cham: Springer.pp.473–501.

Mosgoeller, W., Prassl, R. and Zimmer, A., 2012, Nanoparticle-mediated treatment of pulmonary arterial hypertension. Methods in enzymology, 508: 325-354.

Munishamanna, K.B., Ajey, G., Veena, R. and Palanimuthu, V., 2020, Evaluation of Different Strains of Yeast and Lactic Acid Bacteria for Nutritional Improvement of Jackfruit Waste Under Solid State Fermentation. Mysore Journal of Agricultural Sciences, 54(2): 44-50.

Nguyen, T.T. et al., 2023. Sustainable livestock feed from agricultural residues. Journal of Cleaner Production, 382, 135256.

Patil, V., Chauhan, A. K. and Singh, S. P., 2014, Influence of spray drying technology on the physical and nutritional properties of guava powder. International Journal of Current Microbiology and Applied Sciences, 3(9): 1224-1237.

Pugliese, A., Cabassi, G., Chiavaro, E., Paciulli, M., Carini, E. and Mucchetti, G., 2017, Physical characterization of whole and skim dried milk powders. Journal of food science and technology, 54(11): 3433-3442.

Rahman, M.M. et al., 2021. Nutritional potential of jackfruit by-products. Food Chemistry Advances, 1, 100012.

Ranasinghe, R.A.S.N., Maduwanthi, S.D.T. and Marapana, R.A.U.J., 2019, Nutritional and health benefits of jackfruit (Artocarpus heterophyllus Lam.): a review. International journal of food science, 2019(1), 4327183.

Sarker, M.S.K. et al., 2022. Utilization of fruit waste in livestock feed: A review. Animal       Feed Science and Technology, 290, 115356.

Swami, S. B., Thakor, N. J., Haldankar, P. M. and Kalse, S. B., 2012, Jackfruit and its many functional components as related to human health: a review. Comprehensive Reviews in Food Science and Food Safety, 11(6): 565-576.

Singh, R. et al., 2022. Role of unconventional feed resources in livestock production. Indian Journal of Animal Sciences, 92(4), 456–462.