Review Article

Development of Smart Monitoring Technology in Floating Net Cages to Support sustainable Aquaculture

ABSTRACT

The advancement of digital technology has driven significant transformation in aquaculture systems, particularly in floating net cages (FNC), which operate in dynamic open-water environments. This study aims to synthesize the development of smart monitoring technologies in supporting sustainable aquaculture in FNC systems. A systematic literature review was conducted using 50 relevant studies published between 2019 and 2025, focusing on the application of the Internet of Things (IoT), automated feeding systems, and the integration of machine learning and edge computing. The results indicate that IoT-based systems significantly improve real-time water quality monitoring, enabling faster and more accurate decision-making while reducing environmental risks. Automated feeding systems contribute to increased feed efficiency and reduced organic waste discharge, which is critical in open-water aquaculture systems. Furthermore, the integration of intelligent technologies supports the development of predictive and adaptive aquaculture management. However, several challenges remain, including limitations in sensor accuracy, network stability, and the readiness of human resources in technology adoption. Overall, smart monitoring technologies play a crucial role in enhancing both productivity and environmental sustainability in FNC aquaculture. Future development should focus on more integrated, reliable, and field-adapted systems to support wider implementation in aquaculture practices.

Keywords: smart monitoring; floating net cage; IoT; sustainable aquaculture; automated feeding

# INTRODUCTION

The aquaculture sector plays a vital role in supporting global food security, but it also faces various challenges related to production efficiency and environmental sustainability. One of the rapidly growing aquaculture systems in Indonesia is the floating net cage (FNC), which is typically located in open water such as reservoirs, lakes, and coastal areas. This system offers advantages in utilizing aquatic space but is highly dependent on dynamic environmental conditions, such as fluctuations in water quality, currents, and the accumulation of organic waste from uneaten feed and fish metabolism (Alam et al., 2022; Hossain and Islam, 2021). These conditions necessitate a more precise and adaptive approach to FNC management.

With the advancement of digital technology, the concept of smart aquaculture has emerged as a solution to improve the efficiency and sustainability of aquaculture systems. One widely adopted approach is Internet of Things (IoT) based smart monitoring, which enables real-time and continuous monitoring of water quality. Various studies indicate that the use of IoT in aquaculture can improve monitoring effectiveness and reduce reliance on conventional methods (Megantoro et al., 2024) This technology is particularly relevant for floating net cages located in open water with high levels of environmental variability.

In addition to water quality monitoring, the application of technology has also advanced in operational aspects, such as automated feeding systems. Feeding automation has been shown to improve feed utilization efficiency and reduce waste that could potentially pollute water bodies (Vinoth et al. 2022). In the context of FNC, feed management is a crucial aspect because uneaten feed residues can be directly dispersed into the surrounding environment and contribute to a decline in water quality. Therefore, the integration of water quality monitoring and automated feeding systems is an important part of developing sustainable aquaculture systems.

Further development indicates that smart monitoring technology not only functions as a monitoring tool but is also beginning to integrate with machine learning and edge computing. This integration enables more in-depth data analysis, including environmental condition predictions and early anomaly detection (Deepa and Jeyanthi, 2023; Kaviya et al., 2023; Dharmarathne et al., 2025). This approach supports a shift in aquaculture systems from reactive to more predictive and data-driven, which is critically needed in cage aquaculture system with complex environmental dynamics.

Nevertheless, the implementation of smart monitoring technology in floating net cage still faces various challenges, both technical and non-technical. Some frequently reported obstacles include limited sensor accuracy, network interference in open water locations, and low technology adoption rate among aquaculture farmers (Ananthan et al., 2022; Dharmarathne et al., 2025). Therefore, a comprehensive study is needed to understand existing technological developments while identifying opportunities and challenges in their application.

Based on this background, this study aims to synthesize the development of smart monitoring technology for floating net cages in support of sustainable aquaculture. The results of this study are expected to provide a comprehensive overview of the direction of technological development and serve as a foundation for the development of more efficient, adaptive, and environmentally friendly aquaculture systems.

# METHODOLOGY

This study employs a Systematic Literature Review (SLR) approach to examine the development of smart monitoring technology for floating net cage (FNC) in support of sustainable aquaculture. This approach was chosen because it provides a comprehensive and structured synthesis of various previously published research findings. The review process was conducted in accordance with PRISMA (Preferred Reporting Items for Systematic Reviews and Meta-Analyses) guidelines to ensure transparency, systematicity, and replicability in the literature selection process (Page et al., 2021).

The literature search was conducted through several scientific databases, such as Google Scholar, ScienceDirect, and IEEE Xplore, using combination of relevant keyword, including “smart monitoring”, “Internet of Things”, “floating net cage”, “keramba jaring apung”, “automated feeding”, dan “sustainable aquaculture”. The selected articles were limited to publications from 2019 to 2025 to ensure relevance to current technological developments.

Literature selection was conducted in stages through a process of identification, screening based on titles and abstracts, and evaluation of full-text articles in accordance with the established inclusion and exclusion criteria. Inclusion criteria included articles discussing IoT-based technology, automation, or machine learning in the context of aquaculture, particularly in open water systems such as floating net cages. From this process, 50 articles were identified as relevant and meeting the criteria for further analysis.

Data from each article was then extracted and analyzed using qualitative descriptive methods by grouping the studies into several main themes. IoT based water quality monitoring, automated feeding, smart system integration, and challenges in technology implementation. This approach enables the identification of patterns, trends, and research gaps in the development of smart monitoring in FNC, thereby providing a more comprehensive overview to support the development of sustainable aquaculture (Tranfield et al., 2003).

# RESULTS AND DISCUSSION

Based on the analysis of 50 selected articles, research on the development of smart monitoring technology for floating net cages (FNC) can be grouped into several main themes. This categorization aims to identify the focus of the studies as well as trends in technological development that support sustainable aquaculture. In general, the resulting themes include IoT based water quality monitoring, automated feeding, smart system integration, and challenges in technology implementation. Details of the research theme categorization are presented in Tabel 1.

Table 1. Trend in Smart Monitoring Technology for Floating Net Cage Aquaculture

| No | Research Theme | Focus of Study | Technology Examples | Number of Studies |
| --- | --- | --- | --- | --- |
|  | IoT-based water quality monitoring in floating net cages (FNC) | Monitoring aquatic environmental parameters in real-time in open water FNC systems | Sensor pH, DO sensors, Temperature sensors, Salinity sensors, IOT Platforms | 23 |
|  | Automation of feeding systems in FNC | Efficient feed management to improve productivity and reduce waste | Auto feeder, timer, biomass sensors | 14 |
|  | Integration of intelligent systems (machine learning dan edge computing) | Data processing and utilization to support smart decision-making | Machine learning, edge computing, deep learning | 9 |
|  | Challenges in implementing smart monitoring in FNC | Technical and non-technical constraints in technology application in aquaculture environments | Networking connectivity issues, biofouling, limited human resources | 10 |
|  | Capacity building and enabling factors | The impact of technological implementation on production efficiency and environmental sustainability | Integrated systems, feed efficiency technologies, environmental management tools | 6 |

Source: Authors’ compilation derived from the synthesis of 50 selected articles using the PRISMA framework.

As presented in Table 1, the reviewed studies indicate that IoT-based water quality monitoring is the most dominant research theme in smart monitoring development for floating net cage (FNC) aquaculture systems. This finding highlights the importance of real-time environmental monitoring in maintaining optimal water conditions and supporting fish health in open-water aquaculture environments. Parameters such as pH, dissolved oxygen (DO), temperature, and salinity are the most frequently monitored variables due to their direct influence on fish metabolism, growth, and survival (Boyd, 2020; Føre et al., 2018).

In addition, automated feeding systems represent another major research focus, reflecting the increasing need to improve feed efficiency and reduce operational waste in aquaculture practices. According to Jawad et al. (2020), IoT-based feeding systems can improve feeding accuracy and support more efficient aquaculture management. The integration of intelligent systems, including machine learning and edge computing, further demonstrates the transition toward data-driven and predictive aquaculture systems capable of supporting adaptive decision-making (Abdelrahman et al., 2021).

However, several studies also emphasize the challenges associated with implementing smart monitoring technologies in FNC systems, particularly related to network stability, biofouling, sensor reliability, and limited human resources (Parra et al., 2018; Joffre et al., 2020). These constraints indicate that although smart monitoring technologies have considerable potential to support sustainable aquaculture, further improvements in system integration, infrastructure, and user capacity are still required for broader implementation.

## IoT System Framework in Floating Net Cages (FNC) Aquaculture

The application of Internet of Things (IoT) technology in floating net cage (FNC) aquaculture has become an essential approach to improving production efficiency and environmental sustainability in open-water systems. Unlike conventional monitoring methods, IoT-based systems enable continuous, real-time observation of aquaculture conditions, allowing farmers to respond more quickly and accurately to environmental changes. This is particularly important in FNC systems, where environmental conditions are highly dynamic and directly influence fish health and productivity (Føre et al., 2018; Jawad et al., 2020).

The workflow of IoT-based smart monitoring begins with data acquisition through sensors that continuously measure key water quality parameters such as temperature, pH, dissolved oxygen (DO), and ammonia concentration. These data are transmitted through communication networks, including Wi-Fi, GSM, or LoRa, to cloud-based platforms or local servers. The collected data are then processed and analyzed using IoT platforms, and in more advanced systems, integrated with machine learning algorithms to support predictive analysis and anomaly detection. The results are visualized through dashboards or mobile applications, enabling remote monitoring and data-driven decision-making. Finally, automated responses can be executed through actuators such as auto feeders and aerators, forming a closed-loop system that enhances operational efficiency and reduces manual intervention (Abdelrahman et al., 2021; Parra et al., 2018). The overall workflow of IoT-based smart monitoring in FNC aquaculture is illustrated in Figure 1

Figure 1 Workflow of IoT-based smart monitoring system in floating net cage (FNC)

Source: Developed by the authors based on Føre et al. (2018), Jawad et al. (2020), and Abdelrahman et al. (2021).

The workflow presented in Figure 1 illustrates a closed-loop control system in which real-time data collection and analysis enable adaptive management in aquaculture operations. This approach supports precision aquaculture by allowing timely interventions, such as adjusting feeding strategies or improving aeration based on environmental conditions. As a result, IoT-based systems can reduce operational risks and improve productivity while minimizing environmental impacts (Føre et al., 2018).

In addition to system workflow, smart monitoring technologies in FNC aquaculture can detect a wide range of environmental and biological parameters that are critical for maintaining optimal culture conditions. Water quality parameters, including temperature, dissolved oxygen, pH, salinity, turbidity, and ammonia, are the most important variables influencing fish metabolism, growth, and survival (Boyd, 2020). Environmental parameters such as water currents and weather conditions also play a significant role in open-water aquaculture systems, as they affect nutrient distribution and waste dispersion. Furthermore, recent advancements in smart aquaculture have enabled the monitoring of biological indicators, such as fish behavior and feeding response, through imaging technologies and artificial intelligence, which further enhance decision-making accuracy (Li et al., 2020).

The key parameters monitored in smart aquaculture systems are presented in Table 2.

Table 2 Key parameters monitored in smart aquaculture systems

| Category | Parameter | Function |
| --- | --- | --- |
| Water Quality | Temperature | Affects fish metabolism and growth |
| Water Quality | Dissolved Oxygen (DO) | Essential for respiration |
| Water Quality | pH | Indicates water condition |
| Water Quality | Ammonia | Toxicity indicator |
| Environment | Water Current | Influences waste dispersion |
| Biological | Fish Behavior | Indicates fish health condition |

Source: Compiled by the authors from Boyd (2020), Føre et al. (2018), and Li et al. (2020).

As shown in Table 2, water quality parameters remain the primary focus of IoT-based monitoring systems, as they directly determine aquaculture performance. Among these parameters, dissolved oxygen and temperature are particularly critical in high-density FNC systems, where rapid environmental fluctuations can occur. The inclusion of environmental and biological parameters reflects the advancement of aquaculture toward a more integrated and data-driven management system, commonly referred to as precision aquaculture (Føre et al., 2018; Li et al., 2020).

The effectiveness of IoT-based monitoring systems is influenced by multiple interacting factors, including technical, environmental, and human-related aspects. Technical factors such as sensor accuracy, calibration, biofouling, and network stability are major challenges, especially in open-water environments where conditions are difficult to control. Biofouling, in particular, can significantly reduce sensor performance over time, leading to inaccurate data (Parra et al., 2018). Environmental factors such as water currents, waves, and temperature fluctuations further affect system reliability and data consistency.

In addition, human factors including technological literacy, operational skills, and financial capacity play a critical role in determining the success of IoT adoption in aquaculture systems (Joffre et al., 2020). Without adequate knowledge and resources, the implementation of smart monitoring systems may not be fully optimized. Therefore, successful implementation requires not only technological advancement but also improved system integration and capacity building among aquaculture practitioners.

These influencing factors are summarized in Table 3.

Table 3 Factors affecting the performance of IoT-based monitoring systems in FNC

| Category | Parameter | Function |
| --- | --- | --- |
| Technical | Sensor accuracy | Affects data reliability |
| Technical | Biofouling | Reduce sensor performance |
| Technical | Network stability | Disrupts data transmission |
| Environmental | Water current | Afffects sensor reading |
| Environmental | Temperature fluctuation | Influences fish condition |
| Human | Technical skills | Affects system operation |
| System | Integration level | Determines system efficiency |

Source: Compiled by the authors from Parra et al. (2018), Joffre et al. (2020), and Abdelrahman et al. (2021).

As presented in Table 3, the performance of IoT-based monitoring systems depends on the interaction between various factors. Technical limitations and environmental variability remain key challenges, particularly in FNC systems exposed to open-water conditions. Meanwhile, human and system-related factors determine the level of adoption and effectiveness of technology. Therefore, future development of smart aquaculture systems should focus on improving system robustness, integration, and user accessibility to support sustainable aquaculture practices (Abdelrahman et al., 2021).

## IoT Based Water Quality Monitoring in Floating Net Cages

Based on a review of the literature, the use of the Internet of Things (IoT) has become the primary approach in the development of smart monitoring systems for floating net cages (FNC), which typically operate in open waters (reservoirs, lakes, and the ocean) with highly dynamic environmental conditions. This system enables real-time and continuous monitoring of water quality parameters such as temperature, pH, dissolved oxygen (DO), turbidity, salinity, and ammonia, making it highly relevant for FNS conditions influenced by currents, stratification, and daily fluctuations. Studies by Megantoro et al. (2024), Rahmat et al. (2024), Qomaruddin et al. (2024), Adi et al. (2024), Satra et al. (2023), Ezral et al. (2021), Abdullah et al. (2019), Suhaeri et al. (2020), and Prawara et al. (2022) demonstrate that integrated sensors enhance monitoring effectiveness compared to manual methods. These findings are consistent with Hou et al. (2023), who confirmed improved monitoring efficiency in IoT-based systems.

In the context of cage aquaculture (KJA), IoT systems also function as early warning systems to rapidly respond to drops in dissolved oxygen (DO), temperature spikes, or ammonia increases—phenomena often caused by changes in currents and organic loads around the cages. Several studies indicate that fluctuations in these parameters are directly correlated with fish stress and mortality (Hossain and Islam, 2021; Alam et al., 2022). With real-time monitoring, aqua culturists can immediately take actions such as adjusting stocking density, providing additional aeration, or changing feeding strategies. This underscores the role of IoT in maintaining the stability of cage aquaculture environments, which are vulnerable to rapid changes.

However, technical limitations remain, particularly regarding sensor accuracy in flowing water environments and the need for periodic calibration. Ezral et al. (2021) and Satra et al. (2023) demonstrated that variations in depth, current, and biofouling can affect sensor readings in cage aquaculture. Dharmarathne et al. (2025) also emphasize that sensor degradation in open waters poses a challenge for long-term implementation. Therefore, improving device durability and developing KJA-specific calibration protocols are critical needs.

## Automated Feeding Systems in Floating Net Cages (FNC)

The development of automatic feeding systems is a crucial component of smart monitoring of floating net cages, as it is directly linked to cost efficiency and environmental impact in open waters. Jauhariyanto (2025), Purnama (2025), Yadnya (2023), Pranoto et al. (2024), Deshmukh et al. (2021), H. Santoso et al. (2024), Dwijayanti et al. (2025), Vinoth Kumar et al. (2022), and Huda et al. (2022) demonstrate that automation can precisely regulate feeding frequency and dosage. These findings align with those of Vinoth Kumar et al. (2022), who reported improvements in feed conversion ratio (FCR) efficiency and reductions in operational costs.

In the context of cage aquaculture (FNC), excess feed is immediately dispersed by currents and contributes to increased organic load, local eutrophication, and reduced water quality around the cages. Therefore, precise feed control is key to minimizing waste (Alam et al., 2022). Automated systems allow for feed adjustment based on the actual conditions of the fish and the environment, making them more efficient and environmentally friendly. Current trends indicate the integration of automated feeding systems with water quality sensors in CFCs. Huda et al. (2022) demonstrated that this combination dynamically maintains a balance between feed input and water conditions. This approach aligns with the concept of precision aquaculture, which is highly relevant for CFCs due to their high environmental variability and the need for rapid, data-driven responses.

## Integration of Intelligent Systems : Machine Learning and Edge Computing

Advances in smart monitoring for floating net cages are also leading to the integration of machine learning and edge computing to enhance analytical capabilities in open-water conditions. Kerlooza et al. (2025) demonstrate that edge computing enables local data processing at the floating net pen site, thereby reducing reliance on networks that are often unstable at sea. This is reinforced by Fu and Niu (2023), who state that decentralized processing improves system efficiency and reliability.

Meanwhile, machine learning enables water quality prediction and anomaly detection triggered by changes in currents, weather, and activities around the FNC. Deepa and Jeyanthi (2023) as well as Kaviya et al. (2023) demonstrate that predictive models can estimate water quality trends from historical data, while Ahmed et al. (2023) report improved analysis accuracy through deep learning approaches. Consequently, AQUA systems are evolving from descriptive monitoring to predictive and adaptive systems. However, the main challenge lies in the quality and continuity of data from the dynamic FNC environment. Dharmarathne et al. (2025) emphasize that model performance is highly dependent on stable, interference-free sensor data. Therefore, the integration of reliable hardware and appropriate algorithms is key to the success of smart systems in KJA.

## Challenges in Implementing Smart Monitoring in FNC Systems

The implementation of smart monitoring in floating net cages faces more complex challenges than closed-system aquaculture due to the nature of open waters. Network disruptions, limited energy sources, and exposure to waves and currents are the primary limiting factors (Hou et al., 2023). Additionally, biofouling on sensors in floating net cages accelerates accuracy degradation and increases maintenance requirements.

From a systems perspective, many technologies are still being developed in isolation and have not yet been integrated into a comprehensive platform for FNC. This results in the data generated not being fully utilized in operational decision-making. Ananthan et al. (2022) emphasizes the importance of system integration in the digital transformation of the fisheries sector. Human resources are also a determining factor. Febryanti and Utami (2023) show that aqua culturists’ digital literacy remains a barrier to technology adoption. In FNC, which are often located in remote areas, the need for training and mentoring becomes increasingly important so that the system can be operated and maintained independently.

## Implications for Sustainable Aquaculture in FNC Systems

The implementation of smart monitoring in floating net cages (FNC) provides a significant contribution to sustainable aquaculture, particularly in reducing environmental impacts in open-water systems. This technology enhances production efficiency by minimizing excessive inputs and enabling better control of water quality around the cages. This is important because FNC systems are often associated with issues such as eutrophication and water quality degradation. From a management perspective, data-driven systems enable faster and more adaptive decision-making in response to dynamic environmental conditions in FNC systems. Farmers can optimize stocking density, feeding practices, and responses to fluctuations in water quality with greater precision. This aligns with sustainability principles that emphasize the balance between productivity and environmental conservation (Ananthan et al., 2022).

Therefore, smart monitoring in FNC systems not only serves as a technical support tool but also acts as a foundation for transforming aquaculture into a more modern, adaptive, and environmentally friendly system. In the future, technological development should focus on stronger system integration and be tailored to the specific needs of FNC systems to ensure broader and more sustainable implementation.

# CONCLUSSION

Based on the synthesis of various studies, the development of smart monitoring technology in floating net cage (FNC) aquaculture plays an increasingly important role in supporting sustainable aquaculture. The use of IoT in water quality monitoring has become a key component, as it enables real-time data acquisition and improves the speed and accuracy of decision-making. In addition, automated feeding systems have been proven to enhance operational efficiency while reducing input waste and environmental impacts. The integration of technologies such as machine learning and edge computing further promotes the development of more predictive and adaptive aquaculture systems.

However, the implementation of these technologies still faces several challenges, particularly related to sensor accuracy, network stability, and the readiness of human resources to adopt such innovations. Therefore, future development should focus on improving system reliability, technological integration, and user capacity to ensure the optimal and sustainable implementation of smart monitoring in FNC aquaculture.

COMPETING INTERESTS

The authors would like to express their sincere gratitude to all parties who contributed to the completion of this study. Appreciation is extended to academic supervisors and colleagues for their guidance and valuable input during the research process. The authors also acknowledge that this research did not receive any specific grant from funding agencies in the public, commercial, or not-for-profit sectors. The funding sources had no involvement in the study design, data collection, analysis, interpretation, or writing of the manuscript.

REFERENCE

Abdelrahman, H. A., El-Sayed, A. F. M., & Beck, B. H. (2021). Integration of IoT and actuators for closed-loop systems in aquaculture. Aquacultural Engineering, 92, 102140.

Abdullah, F. A. A., Abidin, N. A. A. Z., Jamil, M. F. A. M., Radzi, N. I. M. R. S. M., & Zaini, N. S. S. M. (2019). Development of smart fish cage monitoring system using IoT. Journal of Physics: Conference Series, 1340(1), 012015

Adi, K., Widodo, A. P., Fitria, A. D. P., & Putranto, A. B. (2024). Design and Implementation of a Shrimp Pond Monitoring System Using Internet Of Things. Journal of Applied Technology, 12(4).

Ahmed, M. A., Hossain, M. S., Rahman, W., Uddin, A. H., & Islam, M. T. (2023). An advanced Bangladeshi local fish classification system based on the combination of deep learning and the internet of things (IoT). Journal of Aquatic Intelligence, 8(3).

Alam, M. A., Hossain, M. B., & Khan, M. F. (2022). Assessment of water quality and plankton diversity of the Meghna River, Bangladesh. Water Resources and Environmental Engineering, 10(2).

Ananthan, S., Renugadevi, K. R. R. R. R., & Abinaya, R. (2022). Digital technology in fisheries  A review. Fisheries Technology Review, 14(1).

Aristho, U. N. K., Arini, A. P., & Leonard, M. D. R. (2025). Sistem Pemantauan Kualitas Air Kolam Berbasis Internet of Things (IoT) untuk Mengurangi Kematian Ikan Nila Menggunakan Logika Fuzzy Mamdani. Jurnal Teknik Informatika, 18(2).

Aziz, M. T., Alam, S. M. R., & Tasmin, F. (2020). Design and Development of an IoT-Based System for Water Quality Monitoring in Fish Ponds. Sensors and IoT Systems, 11(4).

Bobsaid, M. W., & Jaelani, L. M. (2017). Studi Pemetaan Batimetri Perairan Dangkal Menggunakan Citra Satelit Landsat 8 dan Sentinel-2A. Geo-Mapping Journal, 5(2).

Boyd, C. E. (2020). Water Quality: An Introduction. Springer Nature.

Damsir, D., Ansyori, A., Yanto, Y., Erwanda, S., & Purwanto. (2023). Pemetaan areal mangrove di provinsi lampung menggunakan citra sentinel 2-A dan citra satelit google earth. Jurnal Kelautan dan Perikanan, 12(1).

Deepa, P., & Jeyanthi, S. (2023). An Intelligent Monitoring System for Aquaculture Using IoT and Machine Learning. International Journal of Advanced Network Systems, 9(2).

Deshmukh, J. V., Narke, S. S., & Patil, P. R. (2021). IoT-Based Smart Aquaculture Monitoring System Using Wireless Sensor Network. Wireless Communications and Networks, 13(2).

Dharmarathne, G., Abekoon, A. M. S. R., Bogahawaththa, M., Alawatugoda, J., & Meddage, D. P. P. (2025). A review of machine learning and internet-of-things on the water quality assessment: Methods, applications and future trends. Environmental Science & Technology Review, 20(1).

Dwijayanti, S., Suprapto, B. Y., Hikmarika, H., Irmawan, Rendyansyah, Fitria, S., Herlina, & Agustina, S. (2025). Implementasi Teknologi Keramba Jaring Apung Otomatis di Desa Burai. Jurnal Pengabdian Masyarakat Teknik, 7(1).

Ezral, M., Hariyati, E., Othman, M. A., Zulkifli, S. A. B. M., & Diah, A. S. B. A. M. (2021). IoT-Based Multi-Parameters Water Quality Monitoring System for Tilapia Culture. Aquacultural Science and Engineering, 9(3).

Febryanti, D. I., & Utami, D. A. (2023). A hybrid approach towards real-time monitoring of fish distributions in aquaculture scenarios. Journal of Marine and Fishery Economics, 5(1).

Forhad, H. M., Hossain, M. A., & Hossain, M. S. (2024). IoT-based Real-Time Water Quality Monitoring System for Aquaculture. Journal of Smart Systems, 16(1).

Føre, M., Frank, K., Norton, T., Svendsen, E., Alfredsen, J. A., Dempster, T., Eguiraun, H., Handschuh, V., & Berckmans, D. (2018). Precision fish farming: A new framework to improve management in aquaculture. Biosystems Engineering, 173, 176-193.

Fu, X., & Niu, H. (2023). Key technologies and applications of agricultural energy Internet for agricultural planting and fisheries industry. Energy and Agriculture, 5(2).

Hossain, M. B., & Islam, M. S. (2021). A Water Pollution Monitoring System Based on the Internet of Things. Environmental Monitoring Journal, 12(3).

Hou, X., Liu, X., & Zeng, M. (2023). Internet of Things in aquaculture: A review of the challenges and potential solutions based on current and future trends. Sustainable Aquaculture Systems, 11(2).

Huda, N., Azlan, M. N., & Rahim, A. (2022). IoT-Based Automated Fish Feeding and Water Quality Monitoring System. Robotics and Automation in Agriculture, 8(2).

Jamil, F., Semedi, B., & Kurniawan, D. (2020). Pemetaan Batimetri Perairan Dangkal Dengan Menggunakan Citra Satelit Sentinel-2A Dan Landsat 8. Jurnal Penginderaan Jauh, 15(1).

Jauhariyanto, M. (2025). Implementasi Automatic Fish Feeder di Keramba Jaring Apung berbasis IoT. Jurnal Inovasi Teknologi Perikanan, 4(1).

Jawad, H. M., Jawad, A. M., Nordin, R., Gharghan, S. K., Abdullah, N. F., & Ismail, M. (2020). Environmental monitoring systems using wireless sensor networks in smart aquaculture. IEEE Access, 8.

Joffre, O. M., Bosma, R. H., Klerkx, L., & Khatri, S. (2020). Aquaculture innovation system analysis: Thriving in a rapidly changing world. Aquaculture, 520.

Kaviya, S., Kavitha, R., & Gokul, R. (2023). A Hybrid Approach Using IoT and Machine Learning for Smart Aquaculture. Proceedings of the International Conference on Smart Systems.

Kerlooza, Y., Chandra, S. I., & Bachtiar, A. M. (2025). Enhancing IOT Communication Efficacy In Fishery Domain: Edge Computing Perspective. Journal of Computing and Information Technology, 14(2).

Kumar, V. V., Priyadarshini, A., & Buyya, R. (2022). Smart Aquaculture System Based on IoT for Real-Time Monitoring and Feeding Automation. Cloud Computing and IoT Journal, 19(4).

Lin, C. C., Chen, Y. H., & Huang, T. H. (2023). Machine-Learning Estimation of High-Spatio-Temporal-Resolution Chlorophyll-a Concentration Using Multi-Satellite Imagery. Remote Sensing in Aquaculture, 15(2).

Mahestro, D. A., Helmi, M., & Atmodjo, W. (2023). Analisis Perairan Dangkal Berdasarkan Pengolahan Digital Citra Satelit Sentinel-2B di Perairan Pulau Karimunjawa. Jurnal Oseanografi, 10(3).

Megantoro, P., Anugrah, A. W., Abdillah, M. H., Kustanto, B. J., Fadillah, M., & Vigneshwaran, P. (2024). Smart measurement and monitoring system for aquaculture fisheries with IoT-based telemetry system. Aquacultural Engineering Review, 31(3).

Page, M. J., McKenzie, J. E., Bossuyt, P. M., Boutron, I., Hoffmann, T. C., Mulrow, C. D., Shamseer, L., Tetzlaff, J. M., Akl, E. A., Brennan, S. E., Chou, R., Glanville, J., Grimshaw, J. M., Hróbjartsson, A., Lalu, M. M., Li, T., Loder, E. W., Mayo-Wilson, E., McDonald, S., McGuinness, L. A., Stewart, L. A., Thomas, J., Tricco, A. C., Welch, V. A., Whiting, P., & Moher, D. (2021). The PRISMA 2020 statement: An updated guideline for reporting systematic reviews. BMJ, 372.

Parra, L., Sendra, S., Garcia, L., & Lloret, J. (2018). Design and development of a low-cost smart sensor node for water quality monitoring in fish farms. Sensors, 18(9).

Pranoto, D., Nasrullah, E., Sulistiyanti, S. R., & Fitriawan, H. (2024). Rancang Bangun Sistem Pelontar Pakan Ikan Otomatis pada Keramba Jaring Apung. Jurnal Rekayasa Elektro, 10(2).

Prawara, B., Hidayat, N., Tjahyanto, A., Ginting, T. E. K., Mulyanto, B., Harahap, M. A. K. K., et al. (2022). An All-in-One IoT-Based Solution for Monitoring and Automating Tilapia Aquaculture. Information Technology in Fisheries, 14(1).

Purnama. (2025). PkM Pembuatan Alat Pakan Otomatis Berbasis IoT. Jurnal Pengabdian Masyarakat Akuakultur, 6(1).

Qomaruddin, M., Riansyah, A., & Hermawan, H. M. (2024). Mamdani fuzzy-based water quality monitoring and control system in vannamei shrimp farming using the internet of things. Internet of Things in Agriculture Journal, 7(4).

Rahmat, A., Bengen, D. G., Lestari, D. F., Agus, S. B., Pasaribu, R. A., Jusadi, D., & Panggabean, D. (2024). Crab Monitoring System (CMS) using Internet of Things (IoT’s). Marine Science and Technology Bulletin, 15(3).

Santoso, H., Robin, S., Zibar, Z., & Munandar, R. K. (2024). Pengembangan Automatic Fish Feeder untuk Produksi Keramba Apung. Jurnal Teknologi Tepat Guna Perikanan, 8(2).

Satra, R., Hadi, M. S., Sujito, Febryan, Fattah, M. H., & Busaeri, S. R. (2023). IoAT: Internet of Aquaculture Things for Monitoring Water Temperature in Tiger Shrimp Ponds with DS18B20 Sensors and WeMos D1 R2. Journal of Sensor Networks, 11(1).

Setiawan, B., & Surantha, N. (2024). Smart Aquaculture Design for Vannamei Shrimp Farming Based on Quality Function Development. Journal of Information Systems Engineering, 12(1).

Shete, R. P., Bongale, A. M., & Dharrao, D. (2024). IoT-Enabled Effective Real-Time Water Quality Monitoring Method for Aquaculture. IEEE IoT Systems Journal, 10(2).

Silalahi, E., Suprayogi, A., & Sukmono, A. (2018). Studi Pengaruh Keramba Jaring Apung (KJA) Terhadap Kualitas Air di Waduk Kedung Ombo dengan Citra Landsat-8 Multitemporal. Jurnal Geodesi Undip, 7(1).

Sinegar, V. (2010). Pemetaan substrat dasar perairan dangkal. Jurnal Teknologi Perikanan dan Kelautan.

Sitorus, E. (2023). Rancang bangun prototype sistem pemberian pakan dengan Blynk IoT. Jurnal Teknik Informatika.

Suhaeri, S., Irzaman, I., & Bakri, A. S. (2020). Design of Fish Feeds Control System and Water Quality Monitoring Based on IoT. IOP Conference Series: Earth and Environmental Science, 450(1).

Vinoth Kumar, V., Priyadarshini, A., & Buyya, R. (2022). Smart aquaculture system based on IoT for real-time monitoring and feeding automation. Journal of Ambient Intelligence and Humanized Computing.

Wahyu Andini, S., Prasetyo, Y., & Sukmono, A. (2018). Analisis Sebaran Vegetasi dengan Citra Satelit Sentinel Menggunakan Metode NDVI dan Segmentasi. Jurnal Geodesi Undip, 7(3).

Yadnya, I. G. D. A. (2023). Perancangan Alat Pemberian Pakan berbasis IoT. Jurnal Teknik Elektro, 15(2).

Zainal, N. F. A., Ramli, M. K. A. A., Hamid, N. F. A., & Fauadi, M. H. F. M. (2019). Smart fish cage monitoring system using LoRa technology. IEEE International Symposium on Robotics and Intelligent Sensors.

Zhang, L., Chen, X., & Wang, X. (2021). Hybrid IoT Approach for Monitoring Fish Distribution in Net Cage Aquaculture. Journal of Marine Science and Engineering, 9(4).