

1

Minireview Article
MITIGATING FRUITS AND VEGETABLES WASTAGE WITH NEAR-INFRARED (NIR) SPECTROSCOPY
ABSTRACT
Fruits and vegetables are highly perishable, resulting in significant wastage along supply chains. Near-Infrared (NIR) Spectroscopy offers a promising as non-destructive methods for mitigating this issue by providing rapid, accurate, and real-time analysis of the quality and composition of produce. It also enables early detection of spoilage, ripeness, and nutrient levels. This review addresses the gap between controlled validation and real-world deployment by proposing an interdisciplinary framework that integrates food science with supply chain management. The study explores the significance of embedding this technology into the agricultural industry to move beyond basic quality monitoring toward optimized storage and streamlined distribution. By synthesizing these perspectives, this study provides a strategic roadmap for leveraging NIR spectroscopy to enhance monitoring, ultimately mitigating wastage and strengthening food security.
Keywords: Near-Infrared, Spectroscopy, Food security, Post-harvest losses, Non-destructive methods.
Contribution/Originality: This study explores the NIR spectroscopy as system to reduce food wastage and optimize logistics in the supply chain. The originality of study lies in the focus of integrating the technical accuracy of Near-Infrared spectroscopy to a practical framework of reducing post harvest losses for improved food security.
INTRODUCTION
Fruits and vegetables are vital for global food security, offering essential nutrients and supporting to a balanced diet. Extensive research on non-destructive sensors has been conducted over the last few decades (Li et al., 2025). Non-destructive methods for measuring those fruit parameters as stated by Zude et al. (2006) would be beneficial to ascertain the quality of an increased number of fruits individually as well as to monitor the fruit maturity and quality on individual fruit. According to Scott (2020), Near Infra-red (NIR) spectroscopy is a powerful tool that enhances transparency in the fresh produce supply chain and reduces food waste. The technology is also instrumental in controlling quality changes along the supply chain. NIR spectroscopy has been employed for fruit quality assessment for many years (Scott, 2020). 

Nicolai et al. (2014) revealed that the agricultural applications of NIR spectroscopy began by Norris (1964), who used it to measure moisture in grain. Since then, it has been applied for rapid analysis of moisture, protein and fat content in various agricultural and food products (Prempeh et al., 2025). O’Brien and Alamar (2025) noted that the initial applications in horticulture focused on dry matter content of onions, soluble solids content (SSC) of apples and water content in mushrooms, followed by numerous other applications. As highlighted by Bureau et al. (2009), the quality of fruits and vegetables is assessed based on size, color, firmness, surface appearance and other significant sensory properties such as soluble solids and acidity. Hernández-Hierro et al. (2022) noted that in recent decades, mainly benchtop devices have been used to develop at-line methods for qualitative and quantitative control. However, these methods have been limited by the chemical and physical heterogeneity of samples, particularly, those with high moisture content and significant seasonal and regional variations (Hernández-Hierro et al., 2022).
While NIR spectroscopy is well-documented as a tool for laboratory quality analysis, its application in real-world supply chains remains underutilized. This study fills this critical gap by shifting the focus from isolated quality metrics to a holistic, interdisciplinary framework that integrates food science with logistical operations. Driven by recent advancements in miniaturized, high-speed sensing technology, we provide a roadmap for the transition of NIR from the laboratory to the commercial distribution center, offering a scalable solution to the urgent global challenge of food waste. This review intends to explore the capabilities of NIR Spectroscopy technology in reducing wastage of fruits and vegetables. It will cover the significance and fundamental principles of NIR Spectroscopy in evaluating the quality of fresh produce, showcase recent technological advancements, and analyze case studies that illustrate its effectiveness in minimizing food loss. Ultimately, this review aims to emphasize the importance of adopting NIR Spectroscopy as a practical solution to improve the sustainability and efficiency of food supply chain.
Basic Concepts of Near Infrared Spectroscopy
NIR spectroscopy was originally designed for field quality control applications (Hindle, 2008). It was commonly employed for both qualitative and quantitative analysis of agricultural components, including moisture, protein, fat, starch, sugar, fiber, and ash content. The Near-infrared spectral region encompasses wavelengths ranging from 750 nm to 2,500 nm. NIR radiation was discovered and firstly studied by the astronomer Friedrich Wilhelm Herschel in 1800 (Hindle, 2008; Davies, 2000). 
Principles of Near Infrared Spectroscopy in Agriculture
The theoretical potential of Near-Infrared (NIR) spectroscopy in agriculture has been recognized for decades, its practical adoption has historically been hindered by bulky, expensive, and stationary equipment. However, the last five years have witnessed a paradigm shift: the emergence of miniaturized, handheld, and high-speed inline NIR sensors. This technological maturation has finally bridged the gap between laboratory precision and the rugged, high-throughput demands of commercial supply chains, making this the critical moment to evaluate the integration of NIR as a standard industrial practice.

The study reviews an interdisciplinary framework that synthesizes food science; focusing on physiological quality and nutrient retention with supply chain management. By treating the produce not just as a biological entity but as a dynamic asset within a logistical network, this study demonstrates how real-time data can drive automated decision-making, such as dynamic routing and optimized storage conditions, to minimize post-harvest losses.
When radiation hits a sample, the incident radiation may be reflected, absorbed or transmitted, and the relative contribution of each phenomenon depends on the vibrations of the chemical constitution and physical parameters of the sample (Ren et al., 2025). The vibrations’ frequencies determine the parts of the infrared spectrum that they absorb, transmit, or reflect. Thus, each molecule or compound has its unique fingerprint or spectral signature. Based on the spectral signature, it is possible to know the size, shape of atoms and molecules, and the bonds holding them together (Kas et al., 2023). Vijayalaxmi (2024) noted that the information on the structure of the compounds helps to identify the makeup of compounds or plants. Depending on the interaction with light, it is also possible to determine the amount of these compounds. Therefore, infrared light is valuable in studying the different compounds and their concentrations (Vijayalaxmi, 2024). Moreover, plants do not use the infrared spectrum in photosynthesis or any other plant function, so there is no “interference” in producing spectral signatures.

How Near-Infrared (NIR) Works
According to Pandiselvam et al. (2022), near-infrared (NIR) is the light band most suitable for detecting organic compounds because bonds between carbon and hydrogen (C-H), oxygen and hydrogen (O-H), and nitrogen and hydrogen (N-H) absorb wavelengths that fall in the NIR spectrum. Since organic compounds are the major components found in the living tissue of plants and animals, NIR is suitable for use in agriculture and can penetrate deeper into a sample than any other part of the infrared spectrum Vijayalaxmi (2022).
[image: image1.jpg]Wavenumber 2500 cm 4000 cm* 400 cm™* 20cm?
Wavelength 800 nm 2500 nm 25um 500 pum

Molecular vibrations

10° 107 10 1 102 10° 10¢ 108




Fig 1: Spectrum of light showing the NIR region and its wavelength.
Non-Destructive Measurements
The Owino et al. (2024) study on cactus pear fruits focused on the use of non-destructive techniques for measuring quality attributes such as pH, sugar content and firmness etc. The study supports the findings of Alhamdan et al. (2019) stating that one of the requirements for quality control is that measurements have to be non-destructive to save fruit and vegetable loss. The conventional methods, such as an oven or micro-oven method of estimation for dry matter or refractometer for sweetness, are not only destructive, but they are also time-consuming and require training to perform the chemical analyses (Phindile et al., 2022). It also requires sample preparation and space to conduct the tests. According to Scott (2020) NIR spectroscopy has been miniaturized and is now available in small, portable tools that are user-friendly and simple enough to be used by anyone. The results are available instantly and are precise, without variation, due to differences in laboratory conditions.
NIR Spectroscopy Scanner Measurements

Gullifa et al. (2023) explained that NIR spectroscopy is incorporated in many hand-held tools and benchtop devices to provide a wide range of applications, which are increasing with time. The scanner-based tool directs a ray of NIR light of a specified spectrum towards the object. The resulting absorbed, reflected, and transmitted spectra will differ depending on the compounds and their concentrations (Johnson et al., 2023). The scanners in the tools record these spectra. In the case of spectrometers as noted by Li et al. (2023), the spectral signature is used in a wide range of pre-programmed vegetation indices. Most indices use reflectance data, but some will also use absorbance data. 
[image: image2.png]Power supply

Computer

Focusing lens





Fig 2: A set-up of NIRS technology (Non-destructive).
Following products by Felix Applied Food Science Instruments (2026), some NIR spectroscopy devices in the market are:

1. The F-750 Produce Quality Meter can estimate dry matter, soluble solids content (SSC), titratable acidity, and color. It can be used for a wide range of fruits and vegetables.

2. The F-751 Avocado Quality Meter also estimates dry matter and Brix for Haas variety dry matter as an indicator of oil content to determine the maturity of Avocados. It is used to help determine quality of the fruit to determine harvesting as well as post-harvest.
3. The F-751-Mango Quality Meter—which can also estimate dry matter, Brix, and acidity—is customized for mangoes. It has models built for common varieties of mangoes, such as Ataulfo, Tommy K, Kent, Keitt, Calypso, and Honey Gold varieties.
4. The F-751 Kiwi Quality Meter is another quality meter based on the F-750 and is useful to measure maturity and ripeness indicators in the whole supply chain for kiwis.
The Supply Chain

Saechua et al. (2023) stated that NIR spectroscopy is a well-established method to measure quality and maturity indicators, such as dry matter content and total soluble solids. The dry matter at harvest is a good predictor of ripe fruit post-harvest. However, there are many steps between harvest and sale that can also influence fresh produce quality. Once fresh produce is harvested, they are cleaned, sorted, packaged, transported, and treated before they are finally on display in the retail shops as commodities (Paul et al., 2024). The fruits and vegetables are alive, and their characteristics continue to change during this entire process. So, NIR spectroscopy is useful as a quality control measure in all of these phases. An example from Alhamdan and Atia (2017) study showed the ripeness of Barhi dates is evaluated by BRIX, crunchiness, hardness, and development of yellow color. Elamshity and Alhamdan (2025) confirm the use of NIR techniques for identifying the ripening stages in dates using SSC and moisture content. The relation of color with the five stages was less dependable. It is possible to sort the dates using these characteristics (Alhamdan and Atia, 2017).The technology is not used evenly across all categories of fresh produce. Research by Vallone et al. (2019) showed that the extent to which the spectrometers can be used depends on the species and the parameter that is being measured.
[image: image3.jpg]5 P
Hue predicted data

95%Cl  — Fited vales

2 a0
Firmness predicted data




Fig 3: NIR spectroscopy can predict the color far more accurately than firmness in the case of table olives, Vallone et al., 2019. 
Significance in Fruit Ripening

To reduce wastage, the soluble sugar content (SSC), titratable acidity, internal color, and firmness are the parameters of interest in gauging ripeness in fruits.

· SSC: Of all these parameters, SSC is the one that is the industry standard. Besides reflecting fruit physiology, SSC content has proven to be a good indicator of consumer preference (Zude et al., 2006). So, it is used in apple, pear, plum, tomatoes, citrus fruit, kiwi, melon, cherry, etc.

· Titratable acidity: Sweetness is only one of the essential criteria to judge taste and consumer preference. Often a balance between sugars and acids is preferred for the best flavor. Hence, the titratable acidity of many fruits is measured to calculate the sugar to acidity ratio (Peirs et al., 2002). NIR can successfully measure acidity in apple, oranges, tomatoes, peaches, nectarines, and pineapples.

· Firmness: NIR can detect the ideal firmness of ripe fruits in the case of mangoes, tomatoes, and apples (Zude et al., 2006). However, it is less effective in the case of table olives, see Fig 3.
· Color: Internal and external color development is essential to monitor. Here, NIR is suitable for table olives, apples, mango, and tomatoes (Zude et al., 2006).

· Nutrient Content of Fresh Produce: Many of the fresh produce are gaining importance as nutraceuticals due to the high levels of nutritious phytochemicals that they contain. These could be antioxidants, vitamins, polyphenols, pigments like carotenoids, etc. Many fresh produces are tested with NIR spectroscopy to check the concentration of these nutrients (Nadimi and Paliwal, 2024). Produce commonly examined for this reason include blueberries, raspberries, strawberries, banana, carrots, oranges, potatoes, eggplants, olives, etc.

Ripeness parameters are checked several times in the supply chain to sort fruits at different stages. Study by Alvin et al. (2026) confirms that NIR-based tools are useful to sort fresh produce according to their ripeness indicators. For example, when tomatoes are mixed and not sorted, there is a 10% loss of harvest. Ripe or decaying tomatoes produce ethylene, which can prematurely ripen green vegetables in the same pack, spoiling more crop yields. Sorting is carried out again during storage and retailing to remove spoiled produce and reducing losses. Sorting allows for early sales of ripe fruits and later sales of unripe products (Gidado et al., 2026).
Applications of Near Infrared Spectroscopyin Fruits and Vegetables
Applications of NIRS scanner measurements and imaging at various stages in agriculture supply chain as listed by Vijayalaxmi (2022) are;
1. Measurement of Nutritional Status of Crops

NIR scanners are adept at measuring various aspects of plant physiology that reflect the nutritional status of crops. NIR tools provide information on the concentrations, deficiency or excess, of the major nutrients such as nitrogen, phosphorus, and potassium, by detecting chemical composition in leaves (Zahir et al., 2024). NIR testing can replace cumbersome, long, and complicated analysis in the laboratory. An example from Vijayalaxmi (2022) stated that it is often used instead of the comparatively cumbersome Kjeldahl Method to quantify nitrogen in plants. Farmers and agronomists can detect nitrogen deficiency in plants using NIR spectroscopy photosynthesis analysis.

Similarly, NIR technology can detect iron, magnesium, and zinc in leaves of cereal and other crops. The nutritional status of leaves has been determined successfully for apple, citrus, alfalfa, sugarcane, and root crops (Bao et al., 2024). If the analysis shows a deficiency of any element, a farmer can provide timely nutritional supplements in the amounts required. And according to Rawal et al. (2024), they can also avoid over-use of fertilizers, reducing waste and environmental impact.

2. Determining Harvest Time

Digman et al. (2024) demonstrated that one of the most widespread uses of NIR scanner-based devices is measuring dry matter to fix optimum harvest time and monitor the quality of grains, pulses, fruits, and vegetables.
Dry matter (DM) is the total of all solids in a plant minus its water content. DM has been established as one of the most critical parameters in crop production. As emphasized by Islam et al. (2024),non-destructive measurements are necessary since agricultural products are tested several times to determine if they are ready for harvest. Small, cost-effective devices, which give rapid, easy-to-understand results, can sample even large crop areas within a day.
3. NIR for Fruit Processing

Spectroscopic measurements have a wide range of uses during food processing. The use of NIR tools begins on the farm (Bec et al., 2025). Farmers growing fruits meant for processing to make juices, wines, and other liquids, place a premium on sugar content. Fruits have to be harvested at the correct time to achieve the right sugar content. Maturity and ripening are followed meticulously in the field to fix the optimum harvest time (Salehin et al., 2025). The DM content is used to select the harvest date among climacteric fruits, such as apple or mango. In non-climacteric fruits, like berries and grapes that do not ripen much after harvest, the SSC of crops is monitored to fix harvest time. In the case of blackberries, both sugar content and acidity are checked (Gong et al., 2025; Sriti et al., 2025).
There are various NIR applications for sorting and processing fruits and vegetables:

Sugar content is also monitored at post-harvest by using fruits at a similar stage of ripeness for processing.

Water content is measured for several products meant for fresh consumption, such as mushrooms, vegetables, mango, banana, and strawberry.

4. NIR for Detecting Diseases, Pests, and Toxins

Wang et al. (2025) indicated that NIR spectroscopy can be used to detect diseases before visual symptoms appear in growing crops as well as in stored grains. Stress produced due to pests, diseases, and weeds will change plant composition and can also be detected by NIR spectroscopy. Remotely sensed images use this principle in precision agriculture to grade fields based on infection intensity and extent so that growers can make data-driven treatment decisions (Zhu et al., 2024).

· Fusarium (Fusarium asiaticum and Fusarium graminearum) is a common fungal disease that afflicts barley plants, affecting the end quality of the grains and decreasing yield.

· Rice blast caused by Magnaporthe oryzae reduces yield and is a health hazard for people.

Maina and Oerke (2024) demonstrates that hyperspectral reflectance has successfully detected blast fungus using NIR field tools and thus provide a timely and efficient means of controlling infection to limit the damage. During post-harvest storage, rice is particularly susceptible to fungal infections. The research of Ong et al., (2022) showed that NIR technology has been used to find and quantify fungal infections of Aspergillus producing aflatoxins in rice grain. The fungal infection is identified by detecting the presence of aflatoxin B 1 using wavelengths of 950 and 1650 nm.

Reports of Jiang and Wetzel (2025) reveal that NIR technology can also detect insect infestation and cereals’ damage post-harvest by internal compound examination to detect the development of toxins in seeds. For example, the study by Nudel et al. (2024) quantifies amino acid L-canavanine as a toxic compound that can develop in grains and reduce food intake in non-ruminants. The CI-710s SpectraVue Leaf Spectrometer from CID-Science is used to detect plant stress.

5. NIR for Monitoring Drought

Water has its own unique spectral signature that NIR can detect. Ji et al. (2025) developed models in woody plants for measuring leaf water content which lets farmers and agronomists know if the plants are suffering from water stress. They can then use this information to regulate irrigation schedules. The CI-710s SpectraVue Leaf Spectrometer from CID-Science is also an example of a leaf probe widely used for detecting water stress in plants and soil (CID Bio-Science, 2026). 

6. NIR for Quality Control

NIR scanners for sorting, grading, and monitoring the quality of fresh produce and processed products during storage, transport, and retail are a significant application of this technology. Appearance, color, and texture are essential to ensure consumer satisfaction(Feng et al., 2024). Also, NIR testing of DM and SSC levels at harvest allows growers to produce fruits that meet consumer tastes. Elsherbiny et al. (2025) described the early detection of over-ripe produce with NIR imaging, before the appearance of visual symptoms, to help in culling spoiled products in preventing ethylene from spreading and causing early ripening of other nearby produce. Also, ready commercial NIR cameras for spot measurements can be used to inspect the quality of fruits at purchase. The NIR images clearly show bruises long before they are visible to human eyes (Shanthini et al., 2025).Scientists are trying to establish NIR hyperspectral imaging for quality control of walnuts by checking for total fat, monosaturated, saturated, and polyunsaturated fatty acids (Zhao et al, 2025).Similarly, Biswas and Chaudhari (2024) emphasized the role of NIR scanners in quality control of wide-ranging products such as honey and edible oils to ensure good purchases and advise buying choices. By detecting the hydrophilic phenols produced only by olives, NIR testing determines the authenticity of olive oil and checks that it is not adulterated (Li et al., 2025). The F-750 Produce Quality Meter from Felix Instruments (2026) can measure the internal color and firmness of fruits and internal decay in fruits and vegetables.
CONCLUSION
Near-infrared spectroscopy (NIRS) holds significant potential for mitigating fruits and vegetables wastage across the entire supply chain. By providing rapid, non-destructive, and accurate assessment of quality attributes, NIRS enables proactive decision-making, optimization of processes, and value-added enhancements. By harnessing the capabilities of NIRS, farmers or stakeholders can work towards reducing food losses, promoting sustainability, and ensuring food security for current and future generations. Embracing NIRS as a tool for quality assurance and waste reduction represents a tangible step towards a more efficient, resilient, and sustainable food system.
REFERENCES
Alhamdan, A. M., & Atia, A. (2017). Non-destructive method to predict Barhi dates quality at different stages of maturity utilising near-infrared (NIR) spectroscopy. International Journal of Food Properties, 20(sup3). doi: 10.1080/10942912.2017.1387794

Alhamdan, A. M., Fickak, A., &Atia, A. R. (2019). Evaluation of sensory and texture profile analysis properties of stored Khalal Barhi dates nondestructively using Vis/NIR spectroscopy. Journal of Food Process Engineering, 42(6). doi: 10.1111/jfpe.13215 

Alvin, F. P., Wahyunato, A. N., Rifqi, F., Setiyaki, A. N., Harki, H., Aryo, P., Kiki, G., Dwi, S. & Dimas, F. A. R. (2026). Tomato ripeness prediction using low resolution portable spectrometer and machine learning, Spectrochimica Acta Part A: Molecular and Biomolecular Spectroscopy, 344, 126739, 1386-1425, https://doi.org/10.1016/j.saa.2025.126739
Bao, J., Yu, M., Li, J. & Wang, G. (2024). Determination of leaf nitrogen content in apple and jujube by near-infrared spectroscopy. Scientific Reports, 14, 20884. https://doi.org/10.1038/s41598-024-71590-1
Bec, K. B., Grabska, J., & Huck, C. W. (2025). Handheld NIR spectroscopy for real-time on-site food quality and safety monitoring. Advances in food and nutrition research, 115, 293-389. https://doi.org/10.1016/bs.afnr.2025.01.002
Biswas, A., & Chaudhari, S. R. (2024). Exploring the role of NIR spectroscopy in quantifying and verifying honey authenticity: A review. Food Chemistry, 445, 138712. https://doi.org/10.1016/j.foodchem.2024.138712
Bureau, S., Ruiz, D., Reich, M., Gouble, B., Bertrand, D., Audergon, J.-M., and Renard, C.M.G.C. (2009) Rapid and non-destructive analysis of apricot fruit quality using FTnear-infrared spectroscopy, Food Chemistry, 113, 1323

CID Bio-Science. (2026). Spectral leaf measurements for nutrient deficiency detection using the CI-710s SpectraVue Leaf Spectrometer. https://pro-lite.co.uk/products-services-index/spectroscopy/plant-science-tools/cid-plant-science-tools/spectravue-leaf-spectrometer/
Davies, A.M.C. (2000). William Herschel and the discovery of near infrared. Spectrosc. Eur. 12, 10–16.

Digman, M. F., Cherney, J. & Cherney, D. J. R. (2024). Harvest timing of standing corn using near-infrared reflectance spectroscopy. Sensors, 24(5), 1397. https://doi.org/10.3390/s24051397
Elamshity, M. G. &Alhamdan, A. M. (2025). Development and prediction of a non-destructive quality index (Qi) for stored date fruits using VIS-NIR spectroscopy and artificial neural networks. Foods, 14(17), 3060. https://doi.org/10.3390/foods14173060
Elsherbiny, O., El-Hendawy, S., Elsayed, S., et al. (2025). Incorporation of visible/near-infrared spectroscopy and machine learning models for indirect assessment of grape ripening indicators. Scientific Reports, 15, 12345. https://doi.org/10.1038/s41598-024-81694-3
Felix Instruments – Applied Food Science. (2026). F-750 Produce Quality Meter: Non-destructive measurement of internal fruit quality parameters.

Feng, J. T., Yang, Q. Y., Tian, H., Wang, Z. P., Tian, S. J., & Xu, H. R. (2024). Promising real-time fruit and vegetable quality detection technologies applicable to manipulator picking process. International Journal of Agricultural and Biological Engineering, 17(2), 14–26. 10.25165/j.ijabe.20241702.7678

Flinn, P. (2005). An average day (or how near infrared spectroscopy affect daily life). NIRS News 323;16:4-8. 324 

Gidado, M. J., Gunny, A. A. N., Gopinath, S, C. B., Devi, M. & Ibrahim, A. M. (2026). Artificial intelligence and machine learning in postharvest fruit quality assessment: Current challenges, recent advances and future prospects. Journal of Stored Products Research. 116, 102959, https://doi.org/10.1016/j.jspr.2026.102959
Gong, Z., Zhi, Z., Zhang, C., & Cao, D. (2025). Non-destructive detection of soluble solids content in fruits: A review. Chemistry, 7(4), 115. https://doi.org/10.3390/chemistry7040115
Gullifa, G., Barone, L., Papa, E., Giuffrida, A., Materazzi, S., &Risoluti, R. (2023). Portable NIR spectroscopy: the route to green analytical chemistry. Frontiers in Chemistry, 11, 1214825.

Gunasekaran, S. & Irudayaraj, J. (2001). Optical methods: visible NIR and FTIR spectroscopy. In: Nondestructive Food Evaluation. Techniques to Analyse Properties and Quality. Marcel Dekker Inc., New York, USA.

Hernández-Hierro, J.M., Cozzolino, D., Feng, C.H., Rato, A.E. &Nogales-Bueno, J. (2022). Editorial: Recent Advances of Near Infrared Applications in Fruits and Byproducts. Front. Plant Sci. 13:858040. doi: 10.3389/fpls.2022.858040

Hindle, P.H. (2008). Historical development. In D.A. Burns, E.W. Ciurczak (Eds.), Handbook of near-infrared analysis, New York: CRC Press, pp. 3-6.

Islam, M., Bijjahalli, S., Fahey, T. et al. (2024). Destructive and non-destructive measurement approaches and the application of AI models in precision agriculture: a review. Precision Agriculture 25,1127-1180. https://doi.org/10.1007/s11119-024-10112-5
Ji, J., Lu, X., Ma, H., Jin, X., Jiang, S., Cui, H., Lu, X., & Yang, Y. (2025). Estimation of plant leaf water content based on spectroscopy. Frontiers in Plant Science, 16, 1609650. https://doi.org/10.3389/fpls.2025.1609650
Jiang, H., & Wetzel, W. (2025). New AI strategy for mycotoxin detection in cereal grains using near‑infrared spectroscopy. Spectroscopy Online (Food Chemistry).

Johnson, J. B., Walsh, K. B., Naiker, M., & Ameer, K. (2023). The use of infrared spectroscopy for the quantification of bioactive compounds in food: A review. Molecules, 28(7), 3215.

Kas, J. H., Gial, B., Udo, F. H. E., Vasuk, G., David, S. W., Ron, A. W., Jonathan, M. & Jos, O. (2023). An In Silico Infrared Spectral Library of Molecular Ions for Metabolite Identification. Anal. Chem. 95,8998-9005.

Li, C., Czyz, E. A., Halitschke, R., Baldwin, I. T., Schaepman, M. E., & Schuman, M. C. (2023). Evaluating potential of leaf reflectance spectra to monitor plant genetic variation. Plant Methods, 19, 108. https://doi.org/10.1186/s13007-023-01089-9
Li, L., Jia, X., & Fan, K. (2025). Recent advance in nondestructive imaging technology for detecting quality of fruits and vegetables: a review. Critical reviews in food science and nutrition, 65(26), 5181-5199. https://doi.org/10.1080/10408398.2024.2404639
Li, X., et al. (2025). Evaluation of phenolics in the analysis of virgin olive oil using near infrared spectroscopy. Spectrochimica Acta Part A: Molecular and Biomolecular Spectroscopy, 326, 125262. https://doi.org/10.1016/j.saa.2024.125262
Maina, A. W., &Oerke, E.-C. (2024). Hyperspectral imaging for quantifying Magnaportheoryzae sporulation on rice genotypes. Plant Methods, 20, 87. https://doi.org/10.1186/s13007-024-01215-1
Malley, D.&Williams, P. (2005). The future of near infrared spectroscopy: application for the 327 environments. NIRS news; 16:20-2. 328 

Mehinagic, E., Royer, G., Symoneaux, R., Bertrand, D. &Jourjon, F. (2004). Prediction of the sensory quality of apples by physical measurements. Postharvest Biol. Technol. 34, 257–269.

Miyamoto, K., Kawauchi, M., Fukuda, T., 1997. Classification of high acid fruits by PLS using the near infrared transmittance (NIT) spectra of intact Satsuma mandarins. Journal of Near Infrared Spectroscopy 6 (1-4), 267/271.

Nadimi, M. &Paliwal, J. (2024). Recent applications of Near-Infrared spectroscopy in food quality analysis. Foods, 13(16), 2633, https://doi.org/10.3390/foods13162633
Nicolai, B., Defraeye, T., De Ketelaere, B., Herremans, E., Hertog, M., Saeys, W., Torricelli, A., Vandendriessche, T. &Verboven, P. (2014). Nondestructive Measurement of Fruits and Vegetables Quality. Annual review of food science and technology. 5.10.1146/annurev-food-030713-092410

Norris, K.H. (1964). Design and development of a new moisture meter. Agric. Eng. 45, 370.

Nudel, A., Abbo, S., &Kerem, Z. (2024). Canavanine content quantification in processed bitter vetch (Vicia ervilia) seeds and its nutritional implications. Foods, 13(16), 2528. https://doi.org/10.3390/foods13162528
O’Brien, C., &Alamar, M. C. (2025). An Overview of non-destructive technologies for postharvest quality assessment in horticultural crops. The journal of Horticultural Science and Biotechnology, 100(5), 628-636. https://doi/10.1080/14620316.2025.2498874
Ong, P., Tung, I.-C., Chiu, C.-F., Tsai, I.-J., Shih, H.-C., & Chen, S.-H. (2022). Determination of aflatoxin B₁ level in rice (Oryza sativa L.) through near‑infrared spectroscopy and an improved simulated annealing variable selection method. Food Control, 136, 108886. https://doi.org/10.1016/j.foodcont.2022.108886
Owino, W. O., Lanoi, D., Imathiu, S., Kahenya, P., Nyonje, W. A. &Yegon, D. (2024). Near-infrared spectrometry for rapid and real-time prediction of specific quality attributes in intact cactus pear fruits (Opuntia ficus-indica L.). Front. Hortic. 3:1457362. 10.3389/fhort.2024.1457362 

Pandiselvam R, Prithviraj V, Manikantan MR, Kothakota A, Rusu AV, Trif M, Mousavi Khaneghah A. Recent advancements in NIR spectroscopy for assessing the quality and safety of horticultural products: A comprehensive review. Front Nutr. 2022 Oct 12;9:973457. doi: 10.3389/fnut.2022.973457. PMID: 36313102; PMCID: PMC9597448.

Paul, R., Sanyal, D., Malik, A., & Sarkar, T. (2024). Emerging postharvest technologies to enhance the shelf-life of horticultural crops: An overview. International Journal of Advanced Biochemistry Research, 8(4), 49-54. https://doi.org/10.33545/26174693.2024.v8.i4a.907
Peirs, A. et al. (2002). Comparison of Fourier transform and dispersive near-infrared reflectance spectroscopy for apple measurements. Biosystems Engineering.

Phindile, F. N., Lembe, S. M., Samson, Z. T.,&Rebogile, R. M. (2022). Destructive and rapid non-invasive methods used to detect adulteration of dried powdered horticultural products: A review, Food Research International, vol 157, 111198, 0963-9969, https://doi.org/10.1016/j.foodres.2022.111198
Prempeh, N. Y. A., Nunekpeku, X., Kutsanedzie, F. Y. H., Murugesan, A., & Li, H. (2025). A Comprehensive Review of Non-Destructive Monitoring of Food Freshness and Safety Using NIR Spectroscopy and Biosensors: Challenges and Opportunities. Chemosensors, 13(11), 393. https://doi.org/10.3390/chemosensors13110393
Rawal, A., Hartemink, A., Zhangg, Y., Wang, Y. &Ruark, M. (2024). Visible-near infrared spectroscopy for predicting leaf nitrogen content under different management conditions. Precision Agriculture, 25, 751-770. https://doi.org/10.1007/s11119-023-10091-z
Ren, W., Jiang, Q. & Qi, W. (2025). Research progress in near-infrared spectroscopy for detecting the quality of potato crops. Chem. Biol. Technol.Agric. 12, 32. https://doi/10.1186/s40538-025-00747-5
Saechua, S. et al. (2023). Total soluble solids, dry matter content prediction and maturity stage classification of durian fruit using long-wavelength NIR reflectance. Journal of Food Composition and Analysis, 124, 105667. https://doi.org/10.1016/j.jfca.2023.105667

Salehin, M. M., Rafe, M. R. I., Rahman, K. S., Islam, M. R., Ahamed, S., & Rahman, A. (2025). Prediction of sugar content in Java plum using SW-NIR spectroscopy with CNN-LSTM-based hybrid deep learning model. Measurement: Food, 100246. https://doi.org/10.1016/j.meafoo.2025.100246
Shanthini, K. S., Francis, J., George, S. N., George, S., &Devassy, B. M. (2025). Early bruise detection, classification and prediction in strawberry using Vis–NIR hyperspectral imaging. Food Control, 167, 110794. https://doi.org/10.1016/j.foodcont.2024.110794
Sriti, N., Sargent, S., Deng, Z., Williamson, J., & Liu, G. (2025). Postharvest quality and shelf life of three thornless blackberry cultivars over two seasons.Horticulturae, 11(9), 1071. https://doi.org/10.3390/horticulturae11091071
Vallone, M., Alleria, M., Bonob, F., &Cataniaa, P. (2019). Use of a Portable vis Nir Device to Predict Table Olives Quality. Chemical Engineering Transactions, 76: 79-84. DOI: 10.3303/CET1975014
Vijayalaxmi, K. (2022). NIR Applications in Agriculture-Everything You need to know for 2023. A guide to NIRS Applications in Agriculture for 2023.
Vijayalaxmi, K. (2024). Maximizing Agricultural Quality and Efficiency: The Comprehensive Guide to Near-Infrared Spectroscopy (NIRS) Applications. Felix Instruments Applied Food Science.
Wang, Y., Sun, J., Wu, Z., Jia, Y., & Dai, C. (2025). Application of non-destructive technology in plant disease detection: A review.Agriculture, 15(15), 1670. https://doi.org/10.3390/agriculture15151670
Zahir, S. A. D. M., Jamlos, M. F., Omar, A. F., Jamlos, M. A., Mamat, R., Muncan, J. &Tsenkova, R. (2024). Review-Plant nutritional status analysis employing visible and near-infrared spectroscopy spectral sensor. Spectrochimica Acta Part A: Molecular and Biomolecular Spectroscopy, 304, 123273. https://doi.org/10.1016/j.saa.2023.123273
Zhao, Z.,Qiu, J., Liu, X., Chen, M., Cao, W., Chang, S. & Zhao, X. (2025). Combining VNIR and NIR hyperspectral imaging techniques with a data fusion strategy for the determination of fat content, acid value, and storage time of walnuts.Spectrochimica Acta Part A: Molecular and Biomolecular Spectroscopy, 343, 126355. https://doi.org/10.1016/j.saa.2025.126355
Zhu, H., Lin, C., Liu, G., Wang, D., Qin, S., Li, A., Xu, J.-L., & He, Y. (2024). Intelligent agriculture: Deep learning in UAV-based remote sensing imagery for crop diseases and pests’ detection. Frontiers in Plant Science, 15, 1435016. https://doi.org/10.3389/fpls.2024.1435016
Zude. M. et al. (2006). Non-destructive tests on the prediction of apple fruit flesh firmness and soluble solids content on tree and in shelf life. Journal of Food Engineering.


