Methodological Frameworks and Challenges in Product Carbon Footprint Assessment: A Critical Review

Abstract

This paper discusses current methodologies for measuring carbon footprints within the food industry, with a particular focus on seafood and seafood-based products. Key approaches, including life cycle assessment, process modelling, and input-output modelling, are discussed. modelling, and input-output modelling. The carbon footprint methodology estimates total greenhouse gas emissions across the entire the life cycle of a product, from the initial extraction of the raw materials up until the final disposal and serves as essential tools for evaluatingenvironmental consequences and supporting sustainable practices. However, several obstacles exist,  including discrepancies in system boundaries, functional units, regionality, time frames, data availibility, and reliance on emission factors.. These issues complicate identification of hotspots, framework standardization, and limit comparibility of results across studies. Although assumptions and uncertainties may be present, the carbon footprint model serves as a useful instrument for pinpointing sources of emissions and facilitating greenhouse gas reduction in the seafood industry.

Keywords: Carbon footprint, life cycle assessment, greenhouse gases, input-output modelling, system boundaries, GHG Protocol

1. Introduction

Every product or service has an ecological footprint, and it is imperative that the impacts of such products are reduced to help build a more sustainable future, globally grappling with numerous challenges, including resource scarcity and air- and water-quality deterioration, soil pollution, and climate change. Greenhouse gas emissions continue to rise each year, causing an increase in global temperatures (Chang et al. 2017). The concept of a carbon footprint originates from the idea of an ecological footprint, which assesses the consumption of natural resources associated with particular activities (Fang et al. 2014). Research on carbon footprints has been conducted worldwide across diverse fields and regions, encompassing national and economic levels (Munoz et al. 2022; Liang et al. 2022 ), cities and regions (Sovacool and Brown 2010; Wu et al. 2021; Fan et al. 2022),  households (Liao 2022), products (He et al. 2019; Huang et al. 2022), as well as agriculture and aquaculture (Brentrup et al. 2004a; Brentrup et al. 2004b; Chang et al. 2017).

A carbon footprint quantifies the total greenhouse gas emissions produced throughout the entire life cycle of specific products and services, including raw material extraction, production, assembly, transportation, use, disposal, and waste management (Williams 2013). The European Commission (EC) has recognized  Life Cycle Assessment (LCA) as the best framework for evaluating products (Perez-Lopez et al. 2017). LCA has been applied to intensive fish aquaculture systems, including onshore tanks (Jerbi et al. 2012; Iribarren et al. 2012; Samuel-Fitwi et al. 2013) and offshore sea cages (Garcia Garcia et al. 2016; Abdou et al. 2017). It is often noted that comparing LCA results should be avoided (Desjardins et al. 2012; Röös et al. 2013; McAuliffe et al. 2016) due to variations in methodological approaches, functional units, and temporal or regional differences.

The carbon footprint can be quantified per use, per unit, or anually. Even a basic assessment of a product's carbon footprint can serve as a valuable tool in developing a strategy to cut emissions and improve efficiency, leading to cost savings and environmental benefits (Field and Simmons 2012). Establishing clear research objectives and parameters, which encompass core business activities and optional components, is essential (Scrucca 2021). Hence, developing a clear process map that illustrates the flow of materials and energy helps in understanding and enhancing the system being studied (Lee 2011). The main objective of this review is to examine and discuss existing research on standardized methodologies, including life cycle assessment, process analysis, and input-output models, for evaluatinggreenhouse gas emissions across the life cycle of products and organizations.

2. Concept of carbon footprint

The carbon footprint associated with a product represents the quantity of greenhouse gases generated, or utilized over its entire lifecycle. This footprint can be described as conveyed either expressed annually or per use or dosage. It's important to acknowledge that no carbon footprint study can ever be perfect; they are always the best approximation of emissions at a specific moment in time, relying on available data (Berners-Lee 2022). This methodology, in terms of greenhouse gas evaluation has garnered significant interest, particularly due to its alignment with international agreements. In 2008, a Human Development Report was published by the United Nations Development Programme (UNDP) for 2007–2008, emphasizing the requirement for worldwide solutions to confront climate change in the 21st century. An urgent need was felt to confront the challenge of climate change by implementing effective policies to reduce greenhouse gas (GHG) emissions (Fakuda-Parr et al. 2014).

The UNDP report from 2008 indicated that it is not only essential for the world to reach a consensus on GHG emission limits but also to establish national strategies and detail plans for reducing carbon emissions.  Since then, global attention has been focused on climate change, promoting the transition to low-carbon economies and environmentally friendly lifestyles. The goal is to achieve global sustainable development through these low-carbon approaches (Li et al. 2019). In 2009, following discussions at the Copenhagen World Climate Conference regarding the post-Kyoto Protocol commitments, countries emphasized the primary focus on "shared responsibility." They also put forward a long-term global aim of curbing the a 2 °C increase over pre-industrial temperature baselines after the industrial revolution.

An article proposed that if temperatures continue to rise could be restricted to just a rise of 1.5 °C above pre-industrial times, in contrast to the commonly mentioned 2 degrees Celsius, there would be a 75% likelihood targeting a reduction in economic losses of 60% likelihood of bringing in around 20 trillion dollars in economic value (Burke et al.  2018). The Paris Agreement became effective on November 4, 2016. This stands as the third significant legal framework at the international level in the context of mitigating climate change, following the precedents set by the UNFCCC and Kyoto Protocol. These legal tools have formed the foundation for steering international climate governance after 2020 but have also provided a direction for humanity to transition towards new lifestyles and modes pertaining to production. Since the adoption of these agreements, the application and promotion of carbon footprint have gained international recognition (Mancini et al. 2016; Song et al. 2016).

Product carbon footprints (PCFs) are used by businesses to comprehend and control the environmental effects of their products and services (Bolwig and Gibbon 2009). They are motivated by both internal goals, like increasing design efficiency and cutting costs, and external goals, like satisfying customer expectations and enhancing brand image. PCF studies can be resource-intensive despite their benefits, necessitating the careful selection of products with adequate supply-chain visibility as well as precisely defined goals, system boundaries, and life-cycle stages (Mundt and Kemper 2024). Although boundary definitions such as cradle-to-grave or cradle-to-cradle may differ based on the particular objectives of the organization, assessments should ideally take account of the entire life cycle from raw material extraction through production, distribution, use, and end-of-life management. Multiple boundary frameworks, including cradle-to-grave and cradle-to-cradle approaches, may be considered (Figure 2).

3. Methodology for the calculation of carbon footprint

3.1. Assessment standards of carbon footprints

The estimation of the amount of GHGs that get released/are embodied in each step of the process identified within the life cycle of the product, activity, or individual in question is referred to as GHG accounting (Olanrewaju 2025). There are guidelines for GHG accounting. Common resources are:

- GHG Protocol of WRI/WBCSD: Almost all GHG accounting methodologies, such as ISO 14064 and Publicly Available Specifications (PAS) 2050 by British Standards Institution (BSI) in 2008, are founded on the principles laid out in this protocol. The GHG protocol offers distinct protocols for GHG accounting and reporting related to the life cycle of products and organizational structures. ISO 14064 (Parts 1 & 2): ISO 14064 is an international standard prepared by ISO for boundary definition, quantification of greenhouse gas emission, and removal (Manning and Nayak 2024). ISO 14064 part 1 relates to the carbon footprinting of organization with respect to quantification and reporting of GHG emissions, while part 2 covers specific projects and activities.

- Publicly Available Specifications-2050  by the BSI: This sets out the criteria for measuring the GHG emissions in the life cycle of products and services (Bouhmoud et al. 2024). The PAS  standard is working towards developing a methodology to estimate the carbon footprint of agriculture, too.

- International Guidelines for National Greenhouse Gas Inventories by Intergovernmental Panel on Climate Change (IPCC): IPCC classifies all sources of anthropogenic greenhouse gases into four sectors, which include energy, industrial process, agriculture, forestry, and other land use and waste sectors (IPCC 2006).

For the activity for which the carbon footprint is to be calculated, all of these rules and standards go through life cycle assessment  or "cradle-to-grave analyses" (Bassam 2024).

3.2. Selection of GHGs

The selection of greenhouse gases involved in calculations depends on theadopted guidelines , specific requirements for carbon footprinting, and nature of activities undertaken (Yaman 2024). Thus, in thermal power plants, which emit large amounts of CO2, while other gases are insignificantly released into the atmosphere, calculation involving CO2 emissions would only be feasible. In the case of cattle farming, however, CH4, CO2, and N2O emissions could also be relevant. While some literature only considers CO2 emissions in carbon foot printing, all guidelines indicate that all six Kyoto gases should be considered (Iwata and Okada 2014).

3.3. Life Cycle Assessment/ Life Cycle Analysis

Life cycle assessment is a standardized approach that has been adopted and supported by international organizations like the European Commission and the United Nations Environment Program for evaluating the ecological effects of a product throughout its  life cycle process in a way that promotes sustainability and helps in labelling products (Cucurachi et al. 2019). The LCA model consists of four main processes, including goal and scope definition, inventory analysis, impact assessment, and interpretation, which help avoid tradeoffs and minimize burden shifting (Rodrigues and da Silva 2024).

Though there have been many studies conducted using LCA on foods, decision making based on information still poses some difficulties due to differences in methodology, functional units, and temporal/spatial variation, among other reasons, thus restricting their comparisons (Desjardins et al. 2012; Röös et al. 2013; McAuliffe et al. 2016). Other issues include a lack of reviews providing comprehensive information, single value results that do not provide full impact assessment, and lack of public domain LCA information (Peters et al. 2010; Röös et al. 2011).

During the first stage of the life cycle assessment process, the researcher sets out the objectives, scope, assumptions, target audience, and functional unit to help create clarity and validity in carrying out the assessment (Laurent et al. 2020). The functional unit offers an identifiable way of measuring the impact of the product system on the environment by associating it with the functionality of the product, for example, the amount of nutrition supplied by the product system (Vieux et al. 2025).

3.4.  System boundaries and Functional Units

The relevant system boundaries for the carbon footprint study should be established once the comprehensive assessment of the product’s life cycle has been conducted. This analysis helps identify which specific unit processes should be incorporated into the evaluation of the product's climate impact in terms of CO₂-equivalents (Schutzbach et al. 2025). Sample system boundaries for a medium-sized fish fillet processing facility are given in Figure 1 (Poernomo and Suryanto 2020). The raw materials used were sourced from the company’s own farms, including ponds and cages, as well as from external fish farmers. The resulting products were distributed to various cities. During the study, data on quantities of raw materials and products were collected. To estimate the magnitude of CO2 emissions associated with activities within the defined system boundaries, factors translating electricity, fuel, and refrigerant usage into impact metrics in various stages, from originating at the farms and processing plant toward the final retail destination, were employed. Additionally, electricity consumption covering office AC units, office equipment, and lighting systems was computed using the daily mean usage times observed during the study period (Poernomo and Suryanto 2020).

Figure 1.  System boundaries for carbon footprint calculation of pangasius fillet

The study had specific exceptions and limitations in its CO2 analyses, as the analysis did not account for the refrigerant used in office air conditioning units. CO2 emissions related to ice production were calculated based on electricity consumption only, based on assumption that a machine with a capacity of 6 tons/day was used. The calculation did not consider CO2 emissions generated by human labor involved in the production process. Regarding waste treatment, only the emissions from pumps serving liquid wastewater tanks were included, while CO2 emissions from liquid waste within the tank and solid waste were omitted. Fuel consumption for the transportation of raw materials along with product shipments to wholesalers in different cities was estimated according to the total distance travelled using conventional pick-up and refrigerated transport vehicles. Travel distances were approximated using Google Maps, and fuel efficiency data were obtained from the vehicle manual. The study did not track the distribution of products to retailers (Poernomo and Suryanto 2020).

3.5. Data collection and Sources

The problem of collecting data on CO2 equivalent emissions is very difficult because it needs to be collected not just within the organization but from the entire supply chain. The first step here is to find those people who have access to that information and cooperate with them (Stenzel and Waichman 2023).

Data used for product carbon footprint assessment can be categorized into two primary groups, according to Field and Simmons 2012, as given in Table 1.

Table 1. Categories of data used for product carbon footprint

| Primary data | Secondary data |
| --- | --- |
| Measurements and records obtained first hand | Emission factors |
| Typically, this pertains to in-house activities or operations | Sourced from external origins, such as third-party databases |
| For example, quantities like liters of fuel or kilowatt-hours (kWh) of electricity emissions per unit. | For instance, kilograms of CO2 per kilogram of feed at the farm |

Primary data comprises tangible data directly associated with the specific product under examination. Archival data encompasses approximations and postulates derived from recognized industry benchmark or average information. This category also includes emission factors, which play a role in calculations to determine greenhouse gas (GHG) (Field and Simmons 2012).

3.5.1. Primary data

Collecting empirical data for computations of carbon emissions is often formidable because direct measurement is typically not feasible extracted from financial documentation or purchasing specifications. The information needed for this evaluation includes metrics such as weight, distance, and energy consumption, whereas routine documentation primarily focus on fiscal aspects (Chase 2025).

Validated data collection necessitates a firm dedicating time and personnel allocated to the data collection process. Primary data commonly tends to be sourced from multiple divisions of the organization, hence, it is advisable to designate a facilitator to oversee the sequence of steps. Data collection may occur over a specific time frame, typically spanning a year, to mitigate the impact of seasonal fluctuations in the data (Taherdoost 2021).

.a. Utilities: Information pertaining to electricity usage in kilowatt-hours (kWh), the utilization of natural gas in kilowatt-hours (kWh) or cubic meters (m³), and water drawn for use in litres (l) can be sourced from the utility invoices or meter readings. Occasionally, consumption figures can be obtained from sub-metering systems for processes specific to the product, if such systems are in place (Ogola et al. 2023).

Information pertaining to electricity usage in kilowatt-hours (kWh), the utilization of natural gas in kilowatt-hours (kWh) or cubic meters (m³), and water drawn for use in litres (l) can be sourced from the utility invoices or meter readings. Occasionally, consumption figures can be obtained from sub-metering systems for processes specific to the product, if such systems are in place (Ogola et al. 2023).

b. Transport: Information regarding Inbound and outbound freight transportation can be acquired from expense reports, fleet records, fuel details, card records, or structured questionnaires. For estimating Inter-location distances, desk research is often beneficial (Mishra 2024). Expense claims can be explicitly linked to fuel expenses and, consequently, fuel consumption. Assuming the vehicle types are identified, you can multiply mean fuel consumption rates adjusted according to known distances to determine the Cumulative fuel use (Plessis et al. 2023). Fuel use can likewise be computed by conducting surveys of the fleet operations over a representative period. Despite the availability of fuel consumption data which provides greater accuracy, there exists publicly accessible secondary data from Defra that captures the structure of the UK freight system. These data can be utilized to calculate the greenhouse gas impact of transportation when the distance covered is known (Kilian et al. 2023).

Information regarding Inbound and outbound freight transportation is able to be acquired from expense reports, fleet records and fuel details, card records, or structured questionnaires. For estimating Inter-location distances, desk research is often beneficial (Mishra 2024). Expense claims can be explicitly linked to fuel expenses and, consequently, fuel consumption. Assuming the vehicle types are identified, you can multiply mean fuel consumption rates adjusted according to known distances to determine the Cumulative fuel use (Plessis et al. 2023). Fuel use can likewise be computed by conducting surveys of the fleet operations over a representative period. Despite the availability of fuel consumption data which provides greater accuracy, there exists publicly accessible secondary data from Defra that captures the structure of the UK freight system. These data can be utilized to calculate the greenhouse gas impact of transportation when the distance covered is known (Kilian et al. 2023).

. c. Materials: Materials utilized in the fabrication process encompass more than merely the raw materials employed in crafting not only the product but additionally supplementary supplies directly associated with the product's production (Dumee 2022). These auxiliary materials may encompass items like product packaging, fuels that are not included in utility expenses, cleaning agents, and so forth (Kurek and Scetar 2021).

Data pertaining to the constituent parts and mass of these materials is recommended and may typically be acquired from the product specifications, inventory sheets or invoices. In situations where collecting physical data is impractical or impossible, financial data can serve as an alternative source (Navon and Berkovich 2006). This involves Being informed of the materials’ pre-VAT price purchased and then applying factors that link emissions of greenhouse gases in relation to industry sector expenditures. These factors are referred to as Input-Output (I/O) emission factors (Li et al. 2022).

Materials utilized in the fabrication process encompass more than merely the raw materials employed in crafting not only the product but additionally supplementary supplies directly associated with the product's production (Dumee 2022). These auxiliary materials may encompass items like product packaging, fuels that are not included in utility expenses, cleaning agents, and so forth (Kurek and Scetar 2021).

Data pertaining to the constituent parts and mass of these materials is recommended and may typically be acquired from the product specifications, inventory sheets or invoices. In situations where collecting physical data is impractical or impossible, financial data can serve as an alternative source (Navon and Berkovich 2006). This involves Being informed of the materials’ pre-VAT price purchased and then applying factors that link emissions of greenhouse gases in relation to industry sector expenditures. These factors are referred to as Input-Output (I/O) emission factors (Li et al. 2022).

d.Waste: The necessary waste data comprises the material category, quantity of waste by weight, and the method of refuse management employed (e.g., landfill or recycling) (Hoque and Rahman 2020). This information is generally obtainable from records maintained by waste contractors. In cases where such records are unavailable, a waste survey can be conducted over a representative timeframe to ascertain the waste type, weight, and how it is managed (Field and Simmons 2012).

The necessary waste data comprises the material category, quantity of waste by weight, and the method of refuse management employed (e.g., landfill or recycling) (Hoque and Rahman 2020). This information is generally obtainable from records maintained by waste contractors. In cases where such records are unavailable, a waste survey can be conducted over a representative timeframe to ascertain the waste type, weight, and how it is managed (Field and Simmons 2012).

3.5.2. Secondary data

Collecting secondary data for carbon Footprint computation includes conducting research for assembling the required information pertaining to the Product Carbon Footprint (PCF) (Hu et al. 2025).

Secondary data can be categorized into two main types:

a. Secondary Activity Data: This data pertains regarding the quantity required energy, materials, or expenses to execute a specific technique. Examples include the number of kilowatt-hours (kWh) of natural gas needed to produce 1 kilogram of glass or the quantity of kilograms of feldspar required to manufacture 1 kilogram of glass (Falzon et al. 2024).

b. Emission Factors: These data provide information onper-unit environmental impact of a product or activity. For instance, it includes data on the amount of greenhouse gases (GHGs) in kilograms expelled into producing 1 kilogram of glass content or the quantity of GHGs in kilograms released when conveying 1 kilogram of glass for 1 kilometer.

In the case of secondary data, this information is not directly collected from a specific production process; instead, industry averages and approximations are utilized (Legesse 2025).

- To conduct Product Carbon Footprint assessments, it is often necessary to rely on secondary sources, particularly using third-party conversion factors to assess and compute emissions resulting stemming from a particular activity (Hidri 2023). For instance, an enterprise might have data on the quantity of diesel used (activity data), but to ascertain the quantity of carbon emissions generated by burning that diesel, a third-party emission factor is essential (Yu et al. 2022).

- Numerous publicly accessible databases that clarify these emission factors are available, often made available by government departments or international bodies. A valuable preference of these databases can be found on the GHG Protocol’s online platform. This digital portal offers recommendations concerning the process of selecting an appropriate emission factor and provides details on each database, including data type, supported languages, and geographic scope (HosseiniHaghighi et al. 2022).

- The GHG Protocol website highlights databases offering open and accessible resources, such as those from sectoral and national databases, including Plastics Europe, the European Aluminium Association (EAA), Danish Food LCA, and DEFRA emission factors for the UK (Schrijvers et al. 2022). Additionally, through the ELCD database, the European Commission Joint Research Centre supplies freely accessible data from business associations and other sources (Field and Simmons 2012).

- However, it's important to note that some widely used and comprehensive databases, like Ecoinvent and the BUWAL packaging database, are not freely accessible (Bher and Auras 2024).

Reliable secondary sources can be acquired from industry organizations, government reports, regional and national statistical sources, peer-reviewed research, Environmental Product Declarations, databases, verified carbon footprint assessments, Life Cycle Assessments (LCAs), and published data from various other reputable sources (Sajawal Akhtar 2026).

3.6. Quantification of carbon footprint

Greenhouse gas (GHG) emissions are quantified by their weight and transformed into carbon dioxide equivalent emissions through the utilization of 100-year Climate impact potential coefficients, as defined by the Intergovernmental Panel on Climate Change (IPCC). For instance, methane has a GWP (Global Warming Potential) coefficient of 23, signifying that 1 kilogram of methane possesses the same global warming potential as 23 kilograms of carbon dioxide. Greenhouse gas emissions encompass at the level of a 'Product Unit,' which is defined as the item available for purchase by consumers. This 'Product Unit' encompasses the specific packaging used for the product's sale. To facilitate comparisons among  products, carbon emissions can also be expressed as kilograms of CO2 per kilogram of the product (Du et al. 2024).

With respect to carbon footprint evaluation, "hotspots" pertain to the most significant factors or stages that impact resource and energy usage in a product's life cycle, particularly concerning its climate impact. These hotspots serve as valuable indicators for pinpointing critical areas that may require more detailed examination. Hotspots are identified within each life cycle stage (Field and Simmons  2014).

Figure 2: Study boundaries for the main categories of products' carbon footprints

The diagram of the process gives a visual indication of the flows of materials and energy through each stage of the lifecycle of the product, thus aiding in determining the boundaries of the system for carbon emission calculations (Figure 3).

Figure 3: The product process map

To determine the carbon footprint, the methodology outlined in PAS 2050 (British Standards Institution 2011) can be followed. PAS 2050 provides a comprehensive greenhouse gas assessment framework that follows the ISO/TS 14067 guidelines (ISO/TS 14067, 2013). Additionally, PAS 2050 provide  detailed guidance and technical information for conducting the assessment, which enhances the clarity and comprehensibility pertaining to the calculation of a product’s Carbon Footprint. The approach draws on the multiplication of the emission intensity factor associated with a particular material calculated from the amount used in the operation, as outlined in PAS 2050 "activity data." This calculation is represented by the subsequent equation:

CF = EF × MQ

Where:

CF: Carbon Footprint of the material (expressed in X kilograms of CO2 equivalent emissions)

EF: Emission Factor denoting emissions per unit of the material (expressed as X kilograms of CO2 equivalent emissions per 1 kilogram, 1 liter, 1 kilowatt-hour, etc.)

MQ: Quantity of the Material used to complete the entire operation (measured in kilograms, liters, kilowatt-hours, etc.) (Gao et al. 2013).

4. Instruments and Reference databases

Both domestic and international organizations have formulated carbon emissions accounting standards grounded in a comprehensive life-span perspective (Truant et al. 2025). These standards outline the scope, boundaries, and data prerequisites for conducting carbon footprints, while also stipulating the need for consistent and succinct accounting of outcomes, the enactment of emissions cutback measures, and the substantiation process (Kormann and Rodrigues 2025). However, because this document assumes  its users are newcomers to the subject and may not be consulting standards at this juncture, it does not delve into the specific details of these standards. For more information on product footprint standards, you can visit the following websites: BSI, ISO, and GHG Protocol (Field and Simmons 2012).

Table 2.  The carbon footprint associated with seafood products

| Products | Carbon Footprint (kg CO2eq/kg product) | References |
| --- | --- | --- |
| Frozen cod fillet | 2.5 | Ziegler et al. (2016) |
| Frozen salmon fillet | 2.4 | Ziegler et al. (2016) |
| Canned fish | 1.1 | Asakereh et al. (2010) |
| Canned sardines | 7.6 | Almeida et al. (2015) |
| Mahi-mahi, snapper, and wahoo (frozen fillet) | 0.6 g/kg | Sofiah et al. (2018) |

5. Case studies

Carbon footprints represent one of the most politicized fields of science, where there is much at stake due to a considerable degree of uncertainty within the system itself (Constantino and Weber 2021). There have been more case studies of carbon footprinting of farming systems and food in the last few years.

In Liu et al.'s (2023) study on the carbon footprint of large yellow croaker using life cycle assessments of two mariculture models  deep water wind wave-resistant cage (DWWWRC) and traditional plate cage (TPC) ( and ). The total amount of carbon emissions in relation to producing 1,000 kg of large yellow croaker were estimated at 1.055 × 10⁴ kg CO₂-eq.t-1 for DWWWRC and 7.55 × 10⁴ kg CO₂-eq. t-1 TPC. Thediesel  contributed 94.31% and 88.03% to the total emissisions in DWWWRC and TPC,respectively. In addition, according to Ziegler et al. (2021), feed contributes to 85% of the total carbon footprint during salmon production process. In a small-scale Rainbow Trout cage farm in  inland   Turkey, major emission sources included diesel, labor, oxygen, antibiotics, and vitamins. Of these factors, diesel contributed 78.49% of emissions under general management, followed by labor  19.36%, while other inputs such as oxygen (1.66%), antibiotics (0.25%), and vitamins  (0.24%) had relatively lower contributions. . In his study of Hagos (2012), it was foundthat a cage farm raising cobia had the highest carbon footprint with 8 kg CO₂e per kilogram of fish produced. According to Crippa et al. (2021), the GWP for valuenon-ruminant animals such as fish was 3.49 kg CO2eq/kg BFM (Bone Free Meat). According to FAO (2013), the ratio of BFM to fish is 1:0.625. As far as non-ruminant meats go, they have the values of pork (5.77 kg CO2-eq/kg BFM) and chicken (3.65 kg CO2-eq/kg BFM) in that order. According to Ritchie et al. (2023), there are varied values of global warming potential (GWP) for fish in various species like Mackerel (2.00 kg CO2eq/kg BFM), Tuna (2.60 kg CO2eq/kg BFM), Haddock (3.37 kg CO2eq/kg BFM), Sea bass (3.55 kg CO2eq/kg BFM), Pomfret (6.63 kg CO2eq/kg BFM) and salmon (3.76 kg CO2eq/kg BFM). This is in line with Boyd's (2013) report, which found that the CF for each kilogram of fish meat ranged from 2 to 7 kg CO₂e. As pointed out by Halpern et al. (2022), large disparities in results across various species of fish could be attributed to differences in aspects such as the species being farmed, methods adopted in managing the fish farms, farming versus harvesting the fish from the ocean, as well as the way economic allocation is done for those species of fish which are considered high-value. Species harvested from the ocean using trawling and longline fishing vessels generally display high carbon footprint values, owing to high levels of fuel consumption relative to fishing vessels on the coasts (Iribarren et al. 2010; Iribarren et al. 2010a; Vázquez-Rowe et al. 2010b; Vázquez-Rowe et al. 2012). Shellfish like prawns, shrimps, and lobsters are also characterized by wide variations in GWP values, with lobster displaying the highest median value of 27.80 kg CO2-eq/kg BFM.

6. Constraints of the analysis

As this methodology is designed for practical industry applications rather than academic research, it prioritizes key supply chain steps and permits the utilization of secondary data when collecting primary data directly from the supply chain is not feasible (Khan 2025).

Since the primary aim of this methodology is to measure GHG emissions from the main sources, it does not facilitate:

- A detailed comprehension of GHG emissions from each raw material or specific process (Du et al. 2024).

- The identification of GHG emissions from activities that are not typically part of the supply chain process flow, such as activities required only in exceptional cases (Harju et al. 2025).

- The calculation of variations in GHG emissions resulting from all possible alternative supply chain options (Acquaye et al. 2014). This type of analysis is conducted as a secondary step and focuses on the process map for the most common supply chain used by a company for the specific product (Du et al. 2024).

- Inclusion of emissions from the consumption phase is a drawback since this factor is dependent on product types and consumers’ usage behavior (Moran et al. 2020). For products that do not use energy sources, the inclusion of in-use emissions adds to uncertainty without providing any benefit, while for energy-using appliances, in-use emissions are crucial factors that affect consumer decisions. While it is true that in-use emissions matter when it comes to fuels, this methodological approach does not include them since EU measures are being taken regarding this issue (Pavlovic et al. 2018).

- The methodology calculates emissions from cradle to grave, but emissions after labelling are based on average estimates and do not reflect variations in retail practices or consumer disposal methods. It currently excludes emissions at the retail stage, limiting its ability to drive reductions in these areas, though future developments aim to address this gap (Du et al. 2024).

7. Future scope

Research needs to advance carbon footprint assessment by developing better methods that will expand life cycle assessment to cover both cradle-to-grave analyses and regional emission factors. Organizations can track their emissions through direct and indirect methods by using advanced data collection tools which include IoT sensors, smart meters and geospatial techniques. The combination of hybrid LCA models with artificial intelligence and machine learning systems enables organizations to decrease their prediction errors while they gain the ability to forecast how emissions will change over time. The establishment of standardized frameworks together with scenario analysis and sensitivity analysis will enable better assessment of different options, which will help decision makers choose suitable policies that support sustainable development and climate change mitigation efforts.

8. Conclusion

This manuscript primarily emphasizes the research methodology procedures employed in conducting studies related to various types of carbon footprints in relation to products. Additionally, the review explores the objectives, guiding principles, calculation methodologies, data selection processes, and other relevant aspects pertaining to organizational footprints and product carbon footprints. Various standards exist, including Organizational Carbon Footprint Assessment Standards such as ISO 14064 and the Greenhouse Gas (GHG) Protocol, as well as Product Carbon Footprint Assessment Standards like PAS 2050, TSQ0010, ISO 14047, and the Product and Supply Chain GHG Protocol. The most critical components of such study pertain to the selection of greenhouse gases (GHGs), system configurations, quantification and assessment of carbon footprints, data selection, and the treatment of specific releases. These aspects are of utmost significance in the evaluation of carbon footprints for both organizations and products. While existing assessment standards provide guidelines on these matters, there is room for further enhancements and refinements. This paper focuses on the appraisal of the carbon footprint of th seafood in particular and provides an overview for conducting seafood carbon footprint.

Carbon footprint studies are aimed at mitigating the carbon footprint of the seafood from farm to fork, providing a foundation for  future research in this area.

Acknowledgements

The authors are thankful to Prof. Bipul Kumar Das, Faculty of Fishery Sciences, WBUAFS, Chakgaria, Kolkata- 700094, West Bengal, India.

Competing interests

Authors have declared that no competing interests exist.

Authors’ contributions

This work was carried out in collaboration among all authors. All authors read and approved the final manuscript.

References

Abdou, K., Aubin, J., Romdhane, M. S., Le Loc'h, F., & Lasram, F. B. R. (2017). Environmental assessment of seabass (Dicentrarchus labrax) and seabream (Sparus aurata) farming from a life cycle perspective: A case study of a Tunisian aquaculture farm. Aquaculture, 471, 204-212. DOI: https://doi.org/10.1016/j.aquaculture.2017.01.019 Almeida, C., Vaz, S., & Ziegler, F. (2015). Environmental life cycle assessment of a canned sardine product from Portugal. Journal of Industrial Ecology, 19(4), 607-617. DOI: https://doi.org/10.1111/jiec.12219 Asakereh, A., Akram, A., Rafiee, S., & Marzban, A. (2010). Energy consumption and greenhouse gases emission form canned fish production in Iran a case study: Khuzestan province. Research Journal of Applied Sciences, Engineering and Technology, 2(5), 407-413. Bahida, A., Chadli, H., Nhhala, H., Nhhala, I., Wahbi, M., & Erraioui, H. (2022). Carbon Footprint Assessment of a Seabass Farm on the Mediterranean Moroccan Coast. Croatian Journal of Fisheries, 80(4). DOI: https://doi.org/10.2478/cjf-2022-0017 Bassam, H., Lünnemann, P., Riedelsheimer, T., & Lindow, K. (2024). A model-based methodology for life cycle assessment from cradle-to-grave early in product design. Procedia CIRP, 128, 662-667. Berners-Lee, M. (2022). The carbon footprint of everything. Greystone Books Ltd. Bolwig, S., & Gibbon, P. (2009). Emerging product carbon footprint standards and schemes and their possible trade impacts. Danmarks Tekniske Universitet, Risø Nationallaboratoriet for Bæredygtig Energi. Brentrup, F., Küsters, J., Kuhlmann, H., & Lammel, J. (2004). Environmental impact assessment of agricultural production systems using the life cycle assessment methodology: I. Theoretical concept of a LCA method tailored to crop production. European Journal of Agronomy, 20(3), 247-264. DOI: https://doi.org/10.1016/S1161-0301(03)00024-8 Brentrup, F., Küsters, J., Lammel, J., Barraclough, P., & Kuhlmann, H. (2004). Environmental impact assessment of agricultural production systems using the life cycle assessment (LCA) methodology II. The application to N fertilizer use in winter wheat production systems. European Journal of Agronomy, 20(3), 265-279.DOI: https://doi.org/10.1016/S1161-0301(03)00039-X Burke, M., Alampay Davis, W. M., & Diffenbaugh, N. S. (2018). Large potential reduction in economic damages under UN mitigation targets. Nature, 557(7706), 549-553. Chang, C. C., Chang, K. C., Lin, W. C., & Wu, M. H. (2017). Carbon footprint analysis in the aquaculture industry: Assessment of an ecological shrimp farm. Journal of Cleaner Production, 168, 1101-1107. DOI: https://doi.org/10.1016/j.jclepro.2017.09.109 Chase, A. (2025). DEBUNKING" WHAT GETS MEASURED GETS MANAGED": HOW GOVERNMENT PROCUREMENT CAN MOVE PAST CONTRACTOR EMISSIONS DISCLOSURES AND TOWARD ENVIRONMENTAL LIFE CYCLE THINKING. Public Contract Law Journal, 54(4), 835-867. Constantino, S. M., & Weber, E. U. (2021). Decision-making under the deep uncertainty of climate change: The psychological and political agency of narratives. Current opinion in psychology, 42, 151-159. Cucurachi, S., Scherer, L., Guinée, J., & Tukker, A. (2019). Life cycle assessment of food systems. One Earth, 1(3), 292-297. Desjardins, R. L., Worth, D. E., Vergé, X. P., Maxime, D., Dyer, J., & Cerkowniak, D. (2012). Carbon footprint of beef cattle. Sustainability, 4(12), 3279-3301. DOI: https://doi.org/10.3390/su4123279 du Plessis, M. J., van Eeden, J., Goedhals-Gerber, L., & Else, J. (2023). Calculating fuel usage and emissions for refrigerated road transport using real-world data. Transportation Research Part D: Transport and Environment, 117, 103623. Du, J., Zhong, Z., Shi, Q., Cao, Y., Yang, C., & Wang, L. (2024). A review of household carbon footprint: measurement methods, evolution and emissions assessment. Carbon Footprints, 3(3), N-A. DOI: https://doi.org/10.20517/cf.2024.08 Dumée, L. F. (2022). Circular materials and circular design—review on challenges towards sustainable manufacturing and recycling. Circular economy and sustainability, 2(1), 9-23. Falzon, J., Gonzi, R. D., Grima, S., & Vella, E. (2024). Energy performance of buildings for incentivisation in energy-efficient structures: An analysis of secondary data in malta. Processes, 12(5), 874. Fan, W., Huang, S., Yu, Y., Xu, Y., & Cheng, S. (2022). Decomposition and decoupling analysis of carbon footprint pressure in China's cities. Journal of Cleaner Production, 372, 133792. DOI: https://doi.org/10.1016/j.jclepro.2022.133792 Field, B., & Simmons, C. (2012). Product Carbon Footprinting for Beginners. Fukuda-Parr, S., Yamin, A. E., & Greenstein, J. (2014). The power of numbers: a critical review of millennium development goal targets for human development and human rights. Journal of Human Development and Capabilities, 15(2-3), 105-117. Garcia Garcia, B., Rosique Jimenez, C., Aguado-Giménez, F., & Garcia Garcia, J. (2016). Life cycle assessment of gilthead seabream (Sparus aurata) production in offshore fish farms. Sustainability, 8(12), 1228. DOI: https://doi.org/10.3390/su8121228 Garcia-Garcia, G., Azanedo, L., & Rahimifard, S. (2021). Embedding sustainability analysis in new food product development. Trends in Food Science & Technology, 108, 236-244. DOI: https://doi.org/10.1016/j.tifs.2020.12.018 He, B., Liu, Y., Zeng, L., Wang, S., Zhang, D., & Yu, Q. (2019). Product carbon footprint across sustainable supply chain. Journal of Cleaner Production, 241, 118320. DOI: https://doi.org/10.1016/j.jclepro.2019.118320 Hidri, R. (2023). CARBON FOOTPRINT CALCULATOR FOR PRODUCT STRUCTURE DATA. Hoque, M. M., & Rahman, M. T. U. (2020). Landfill area estimation based on solid waste collection prediction using ANN model and final waste disposal options. Journal of Cleaner Production, 256, 120387. HosseiniHaghighi, S., de Uribarri, P. M. Á., Padsala, R., & Eicker, U. (2022). Characterizing and structuring urban GIS data for housing stock energy modelling and retrofitting. Energy and Buildings, 256, 111706. Schrijvers, D., Bolle, C., & Ferrieres, M. (2024). Guide to accounting for avoided GHG emissions in the waste recovery and recycling sector (version 2.0) (Doctoral dissertation, RECORD). Bher, A., & Auras, R. (2024). Life cycle assessment of packaging systems: A meta-analysis to evaluate the root of consistencies and discrepancies. Journal of Cleaner Production, 476, 143785. Sajawal Akhtar, M., Gómez, F. R., Pandit, A., Mahmood, H., Khan, M. A., & Liu, J. (2026). Toward standards-compliant dynamic life cycle assessment: harnessing digital innovations for climate and regulatory alignment. ACS Sustainable Chemistry & Engineering, 14(11), 5295-5314. Truant, E., Crocco, E., Corazza, L., & Borlatto, E. (2025). Life cycle thinking and carbon accounting in sustainable supply chains: a structured literature review and research agenda. Sustainability Accounting, Management and Policy Journal, 16(5), 1370-1393. Kormann, J. P. G., & Rodrigues, W. (2025). Sustainable Accounting: Impact of the carbon footprint: Accounting Strategies and Risk and Opportunity Management in the Carbon Footprint Era. Jackson Patrick Gleich Kormann. Harju, A., Karttunen, E., & Hallikas, J. (2025). Understanding the systemic sources of uncertainty for the management of greenhouse gas emissions in supply chains. International Journal of Operations & Production Management, 45(11), 1884-1909. Acquaye, A., Genovese, A., Barrett, J., & Lenny Koh, S. C. (2014). Benchmarking carbon emissions performance in supply chains. Supply Chain Management: An International Journal, 19(3), 306-321.. Moran, D., Wood, R., Hertwich, E., Mattson, K., Rodriguez, J. F., Schanes, K., & Barrett, J. (2020). Quantifying the potential for consumer-oriented policy to reduce European and foreign carbon emissions. Climate Policy, 20(sup1), S28-S38.

Hu, J., Shi, Y., Cheng, Y., & Liu, Z. (2025). A process-oriented model to measure product carbon footprint: an exploratory study based on multiple cases. Production Planning & Control, 36(3), 285-308.

Huang, W., Wu, F., Han, W., Li, Q., Han, Y., Wang, G., ... & Wang, Z. (2022). Carbon footprint of cotton production in China: Composition, spatiotemporal changes and driving factors. Science of the Total Environment, 821, 153407. DOI: https://doi.org/10.1016/j.scitotenv.2022.153407

Ipcc, I. P. C. C. (2006). Guidelines for national greenhouse gas inventories. Prepared by the National Greenhouse Gas Inventories Programme. Eggleston HS, Buendia L, Miwa K, Ngara T, Tanabe K, editors. Published: IGES, Japan.

Iribarren, D., Moreira, M. T., & Feijoo, G. (2010). Revisiting the life cycle assessment of mussels from a sectorial perspective. Journal of Cleaner Production, 18(2), 101-111. DOI: https://doi.org/10.1016/j.jclepro.2009.10.009

Iribarren, D., Moreira, M. T., & Feijoo, G. (2012). Life cycle assessment of aquaculture feed and application to the turbot sector.

Iribarren, D., Vázquez-Rowe, I., Hospido, A., Moreira, M. T., & Feijoo, G. (2010). Estimation of the carbon footprint of the Galician fishing activity (NW Spain). Science of the Total Environment, 408(22), 5284-5294. DOI: https://doi.org/10.1016/j.scitotenv.2010.07.082

Iwata, H., & Okada, K. (2014). Greenhouse gas emissions and the role of the Kyoto Protocol. Environmental Economics and Policy Studies, 16(4), 325-342.

Jerbi, M. A., Aubin, J., Garnaoui, K., Achour, L., & Kacem, A. (2012). Life cycle assessment (LCA) of two rearing techniques of sea bass (Dicentrarchus labrax). Aquacultural Engineering, 46, 1-9. DOI: https://doi.org/10.1016/j.aquaeng.2011.10.001

Kilian, L., Owen, A., Newing, A., & Ivanova, D. (2023). Microdata selection for estimating household consumption-based emissions. Economic Systems Research, 35(3), 325-353.

Kurek, M., & Ščetar, M. (2021). Auxiliary Materials. Packaging Materials and Processing for Food, Pharmaceuticals and Cosmetics, 235-256.

Laurent, A., Weidema, B. P., Bare, J., Liao, X., de Souza, D. M., Pizzol, M., ... & Verones, F. (2020). Methodological review and detailed guidance for the life cycle interpretation phase. Journal of industrial ecology, 24(5), 986-1003.

Lee, K. H. (2011). Integrating carbon footprint into supply chain management: the case of Hyundai Motor Company (HMC) in the automobile industry. Journal of cleaner production, 19(11), 1216-1223. DOI: https://doi.org/10.1016/j.jclepro.2011.03.010

Legesse, B. (2025). CARBON FOOTPRINT AND ITS MITIGATION STRATEGIES IN JEMO GLASS INDUSTRY, ADDIS ABABA.

Li, J., Liu, H., & Du, K. (2019). Does market-oriented reform increase energy rebound effect? Evidence from China's regional development. China Economic Review, 56, 101304. DOI: https://doi.org/10.1016/j.chieco.2019.101304

Li, S., Wang, W., Diao, H., & Wang, L. (2022). Measuring the efficiency of energy and carbon emissions: A review of definitions, models, and input-output variables. Energies, 15(3), 962.

Liang, D., Lu, H., Guan, Y., & Feng, L. (2022). Drivers for decoupling carbon footprint pressure from economic growth in China’s provinces. Geography and Sustainability, 3(3), 258-267. DOI: https://doi.org/10.1016/j.geosus.2022.08.003

Liao, M., Jia, J., Han, G., Wang, G., Zhang, Z., & Wu, G. (2023). Reduced carbon footprint inequality in China: Evidence from latest household survey data. Journal of Cleaner Production, 384, 135342. DOI: https://doi.org/10.1016/j.jclepro.2022.135342

Lu, H., & Lu, H. (2026). Empirical Lifecycle Assessment of Generative AI Inference Carbon Emissions: Global Trends and the Malaysian'Carbon Arbitrage'Risk (2023–2030). Uniglobal Journal of Social Sciences and Humanities, 5(1), 133-144.

Mancini, M. S., Galli, A., Niccolucci, V., Lin, D., Bastianoni, S., Wackernagel, M., & Marchettini, N. (2016). Ecological footprint: refining the carbon footprint calculation. Ecological indicators, 61, 390-403. DOI: https://doi.org/10.1016/j.ecolind.2015.09.040

Manning, L., & Nayak, R. (2024). Carbon farming: Standards, good agricultural practices, reporting and verification activities. Improving standards and certification in agri-food supply chains.

McAuliffe, G. A., Chapman, D. V., & Sage, C. L. (2016). A thematic review of life cycle assessment (LCA) applied to pig production. Environmental Impact Assessment Review, 56, 12-22. DOI: https://doi.org/10.1016/j.eiar.2015.08.008

Mishra, A. K. (2024). Documentation in logistics. In Supply Chain Management (pp. 212-230). CRC Press.

Mundt, J., & Kemper, M. (2024). The challenges of PCF creation in the automotive value chain: Harmonisation proposals for the GHG accounting of products. Hamburg Institut Discussion Papers.

Muñoz, M., Reul, A., Guijarro, B., & Hidalgo, M. (2023). Carbon footprint, economic benefits and sustainable fishing: Lessons for the future from the Western Mediterranean. Science of the Total Environment, 865, 160783. DOI: https://doi.org/10.1016/j.scitotenv.2022.160783

Navon, R., & Berkovich, O. (2006). An automated model for materials management and control. Construction Management and Economics, 24(6), 635-646.

Ogola, N., Musau, P. M., & Wekesa, C. (2023). Assessment of the implementation of energy conservation opportunities arising from energy audits; A study of four-star and five-star hotels in Nairobi Kenya.

Pavlovic, J., Anagnostopoulos, K., Clairotte, M., Arcidiacono, V., Fontaras, G., Rujas, I. P., ... & Ciuffo, B. (2018). Dealing with the gap between type-approval and in-use light duty vehicles fuel consumption and CO2 emissions: present situation and future perspective. Transportation Research Record, 2672(2), 23-32.

Perez-Lopez, P., Ledda, F. D., Bisio, A., Feijoo, G., Perino, E., Pronzato, R., ... & Moreira, M. T. (2017). Life cycle assessment of in situ mariculture in the Mediterranean Sea for the production of bioactive compounds from the sponge Sarcotragus spinosulus. Journal of Cleaner Production, 142, 4356-4368. DOI: https://doi.org/10.1016/j.jclepro.2016.11.137

Peters, G. M., Rowley, H. V., Wiedemann, S., Tucker, R., Short, M. D., & Schulz, M. (2010). Red meat production in Australia: life cycle assessment and comparison with overseas studies. Environmental science & technology, 44(4), 1327-1332. DOI: https://doi.org/10.1021/es901131e

Poernomo, A., & Suryanto, M. R. (2020). Carbon footprint of frozen pangasius fillet: a case study. In IOP Conference Series: Earth and Environmental Science (Vol. 404, No. 1, p. 012001). IOP Publishing. DOI:  https://doi.org/10.1088/1755-1315/404/1/012001

Rodrigues, S. L., & da Silva, E. A. (2024). Implementation of environmental life cycle costing: Procedures, challenges, and opportunities. The International Journal of Life Cycle Assessment, 29(5), 803-837.

Röös, E., Sundberg, C., Tidåker, P., Strid, I., & Hansson, P. A. (2013). Can carbon footprint serve as an indicator of the environmental impact of meat production?. Ecological indicators, 24, 573-581. DOI: https://doi.org/10.1016/j.ecolind.2012.08.004

Samuel-Fitwi, B., Schroeder, J. P., & Schulz, C. (2013). System delimitation in life cycle assessment (LCA) of aquaculture: striving for valid and comprehensive environmental assessment using rainbow trout farming as a case study. The International Journal of Life Cycle Assessment, 18(3), 577-589. DOI: https://doi.org/10.1007/s11367-012-0510-z.

Scrucca, F., Barberio, G., Fantin, V., Porta, P. L., & Barbanera, M. (2020). Carbon footprint: Concept, methodology and calculation. In Carbon footprint case studies: Municipal solid waste management, sustainable road transport and carbon sequestration (pp. 1-31). Singapore: Springer Singapore.

Sofiah, I., Yani, M., & Ismayana, A. (2018). Dampak pemanasan globalpengolahan hasil perikanan menggunakan metodelife cycle assessment (LCA): analisis gate-to-gate. Jurnal Teknologi Industri Pertanian, 28(1).

Song, G., Che, L., & Zhang, S. (2016). Carbon footprint of a scientific publication: A case study at Dalian University of Technology, China. Ecological Indicators, 60, 275-282. DOI: https://doi.org/10.1016/j.ecolind.2015.06.044

Sovacool, B. K., & Brown, M. A. (2010). Twelve metropolitan carbon footprints: A preliminary comparative global assessment. Energy policy, 38(9), 4856-4869. DOI: https://doi.org/10.1016/j.enpol.2009.10.001

Stenzel, A., & Waichman, I. (2023). Supply-chain data sharing for scope 3 emissions. Npj Climate Action, 2(1), 7.

Taherdoost, H. (2021). Data collection methods and tools for research; a step-by-step guide to choose data collection technique for academic and business research projects. International journal of academic research in management (IJARM), 10(1), 10-38.

Vázquez-Rowe, I., Moreira, M. T., & Feijoo, G. (2010). Life cycle assessment of horse mackerel fisheries in Galicia (NW Spain): comparative analysis of two major fishing methods. Fisheries Research, 106(3), 517-527. DOI: https://doi.org/10.1016/j.fishres.2010.09.027

Vázquez-Rowe, I., Moreira, M. T., & Feijoo, G. (2012). Inclusion of discard assessment indicators in fisheries life cycle assessment studies. Expanding the use of fishery-specific impact categories. The International Journal of Life Cycle Assessment, 17(5), 535-549. DOI: https://doi.org/10.1007/s11367-012-0395-x

Vieux, F., Maillot, M., Marmonier, C., Rouault, A., Perignon, M., & Darmon, N. (2025). Relative environmental impacts and monetary cost of food categories: Functional unit matters. Ecological Economics, 234, 108620.

Williams, D. R. (2013). Climate Change Influence on Building Lifecycle Greenhouse Gas Emissions (Volume I). University of Surrey (United Kingdom).

Wu, X., Li, C., Guo, J., Wu, X., Meng, J., & Chen, G. (2021). Extended carbon footprint and emission transfer of world regions: With both primary and intermediate inputs into account. Science of The Total Environment, 775, 145578. DOI:  https://doi.org/10.1016/j.scitotenv.2021.145578

Yaman, C. (2024). A review on the process of greenhouse gas inventory preparation and proposed mitigation measures for reducing carbon footprint. Gases, 4(1), 18-40.

Yu, X., Zheng, H., Sun, L., & Shan, Y. (2020). An emissions accounting framework for industrial parks in China. Journal of Cleaner Production, 244, 118712.

Ziegler, F., Hornborg, S., Green, B. S., Eigaard, O. R., Farmery, A. K., Hammar, L., ... & Smith, A. D. (2016). Expanding the concept of sustainable seafood using Life Cycle Assessment. Fish and Fisheries, 17(4), 1073-1093.  DOI: https://doi.org/10.1111/faf.12159

Olanrewaju, A. (2025). Net Zero Within the Built Environment. In Sustainable Construction Management: Research and Practice Companion: Integrating Sustainability Target in Construction (pp. 199-249). Singapore: Springer Nature Singapore.

Bouhmoud, H., Loudyi, D., & Azhar, S. (2024). Building information modeling (BIM) for lifecycle carbon emission: scientometric and scoping literature reviews. Smart and Sustainable Built Environment, 13(6), 1349-1369.

Schutzbach, M., Miehe, R., & Sauer, A. (2025). Simplifying life cycle Assessment: Basic considerations for approximating product carbon footprints based on corporate carbon footprints. Ecological Indicators, 176, 113710.

Gao, T., Liu, Q., & Wang, J. (2014). A comparative study of carbon footprint and assessment standards. International Journal of Low-Carbon Technologies, 9(3), 237-243.

Liu, J., Gui, F., Zhou, Q., Cai, H., Xu, K., & Zhao, S. (2023). Carbon footprint of a large yellow croaker mariculture models based on life-cycle assessment. Sustainability, 15(8), 6658.

Ziegler, F., Jafarzadeh, S., Hognes, E. S., & Winther, U. (2022). Greenhouse gas emissions of Norwegian seafoods: From comprehensive to simplified assessment. Journal of Industrial Ecology, 26(6), 1908-1919.

Hagos, K. W. (2012). Survey of resource use efficiency and estimation of carbon and water footprints in fish farming systems using life cycle analysis. University of Rhode Island.

Hagos, K. W. (2012). Survey of resource use efficiency and estimation of carbon and water footprints in fish farming systems using life cycle analysis. University of Rhode Island.

FAO 2013.        https://www.fao.org/3/i3460e/i3460e05.pdf (Accessed on : 14.02.24)

Ritchie, H., Rosado, P., & Roser, M. (2020). Greenhouse gas emissions. Our world in data.

Khan, S. A. R., Godil, D. I., Jabbour, C. J. C., Shujaat, S., Razzaq, A., & Yu, Z. (2025). Green data analytics, blockchain technology for sustainable development, and sustainable supply chain practices: evidence from small and medium enterprises. Annals of Operations Research, 350(2), 603-627.